Annex A (normative):
RRM test configurations

# A.1	Reference measurement channels

This clause contains the Reference Measurement Channels (RMC) to be used for the RRM test scenarios in Clauses 4 to 7 of this document.

## A.1.1	PDSCH

### A.1.1.1	FDD

Table A.1.1.1-1: PDSCH Reference Measurement Channels for SCS = 15 kHz for FDD

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | SR.1.1 FDD |
| Channel bandwidth | MHz | Defined in test case |
| Number of transmitter antennas |  | 1 |
| Allocated resource blocks for PDSCH Note 1 |  | 24 |
| Allocated slots per Radio Frame |  | 10 |
| Radio frame containing SSB | slots | Note 5 |
| Radio frame not containing SSB | slots | 10 |
| MCS index |  | 4 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 1/3 |
| Number of control symbols |  | 2 |
| PDSCH mapping type |  | Type A |
| Information Bit Payload |  |  |
| For slots with RMSI Note 2 | Bits | 1608 |
| For slots without RMSI | Bits | 1864 |
| Number of Code Blocks per slot |  | 1 |
| Binary Channel Bits Per slot |  |  |
| For slots with RMSI Note 2, 4 | Bits | 5184 |
| For slots without RMSI Note 6 | Bits | 6048 |
| NOTE 1: Allocated outside the SMTC duration in time and in resource blocks which do not overlap with the resource blocks allocated for SS/PBCH block. NOTE 2: PDSCH is scheduled on the slots with RMSI. NOTE 3: If necessary, the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in 3GPP TS 38.213 [8]. NOTE 4: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 2. NOTE 5: PDSCH is not scheduled in slots containing SSB according to the SSB configuration used in the test. SSB configurations are defined in clause A.3. NOTE 6: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1. |  |  |

### A.1.1.2	TDD

Table A.1.1.2-1: PDSCH Reference Measurement Channels for SCS = 15 kHz for TDD

| Parameter | Unit | Value | Value |
| --- | --- | --- | --- |
| Reference channel |  | SR.1.1 TDD | SR.1.2 TDD |
| Channel bandwidth | MHz | Defined in test case | Defined in test case |
| Number of transmitter antennas |  | 1 | 1 |
| Allocated resource blocks for PDSCH Note 1 |  | 24 | 24 |
| Allocated slots per Radio Frame |  |  |  |
| Radio frame containing SSB | slots | Note 5 | Note 5 |
| Radio frame not containing SSB | slots | 4 | 6 |
| MCS table |  | 64QAM | 64QAM |
| MCS index |  | 4 | 4 |
| Modulation |  | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 |
| Number of control symbols |  | 2 | 2 |
| PDSCH mapping type |  | Type A | Type A |
| Information Bit Payload |  |  |  |
| For slots with RMSI Note 2 | Bits | 1608 | 1608 |
| For slots without RMSI | Bits | 1864 | 1864 |
| For special slots | Bits | N/A | 1128 |
| Number of Code Blocks per slot |  | 1 |  |
| Binary Channel Bits Per slot |  |  |  |
| For slots with RMSI Note 2, 4 | Bits | 5184 | 5184 |
| For slots without RMSI Note 6 | Bits | 6048 | 6048 |
| For special slots Note 6 | Bits | - | 3744 |
| NOTE 1: Allocated outside the SMTC duration in time and in resource blocks which do not overlap with the resource blocks allocated for SS/PBCH block. NOTE 2: PDSCH is scheduled on the slots with RMSI. NOTE 3: If necessary, the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in 3GPP TS 38.213 [8]. NOTE 4: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 2. NOTE 5: PDSCH is not scheduled in slots containing SSB according to the SSB configuration used in the test. SSB configurations are defined in clause A.3.10. NOTE 6: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1. |  |  |  |

Table A.1.1.2-2: PDSCH Reference Measurement Channels for SCS = 30 kHz for TDD

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | SR.2.1 TDD |
| Channel bandwidth | MHz | Defined in test case |
| Number of transmitter antennas |  | 1 |
| Allocated resource blocks for PDSCH Note 1 |  | 24 |
| Allocated slots per Radio Frame |  |  |
| Radio frame containing SSB | slots | Note 5 |
| Radio frame not containing SSB | slots | 10 |
| MCS table |  | 64QAM |
| MCS index |  | 4 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 1/3 |
| Number of control symbols |  | 2 |
| PDSCH mapping type |  | Type A |
| Information Bit Payload |  |  |
| For slots with RMSI Note 2 | Bits | 1608 |
| For slots without RMSI | Bits | 1864 |
| Number of Code Blocks per slot |  | 1 |
| Binary Channel Bits Per slot |  |  |
| For slots with RMSI Note 2, 4 | Bits | 5184 |
| For slots without RMSI Note 6 | Bits | 6048 |
| NOTE 1: Allocated outside the SMTC duration in time and in resource blocks which do not overlap with the resource blocks allocated for SS/PBCH block. NOTE 2: PDSCH is scheduled on the slots with RMSI. NOTE 3: If necessary, the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in 3GPP TS 38.213 [8]. NOTE 4: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 2. NOTE 5: PDSCH is not scheduled in slots containing SSB according to the SSB configuration used in the test. SSB configurations are defined in clause A.3.10. NOTE 6: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1. |  |  |

Table A.1.1.2-3: PDSCH Reference Measurement Channels for SCS = 120 kHz for TDD

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | SR.3.1 TDD | SR.3.2 TDD | SR.3.3 TDD |
| Channel bandwidth | MHz | 100 | 100 | 100 |
| Number of transmitter antennas |  | 1 | 1 | 1 |
| Allocated resource blocks for PDSCH |  | 24Note 1 | 24Note 7 | 48Note 7 |
| Allocated slots per Radio Frame |  |  |  |  |
| Radio frame containing SSB | slots | Note 5 | Note 8 | Note 5 |
| Radio frame not containing SSB | slots | 48 | 48 | 48 |
| MCS table |  | 64QAM | 64QAM | 64QAM |
| MCS index |  | 4 | 4 | 4 |
| Modulation |  | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 |
| Number of control symbols |  | 2 | 2 | 2 |
| PDSCH mapping type |  | Type A | Type A | Type A |
| Information Bit Payload |  |  |  |  |
| For slots with RMSI | Bits | 1608 | 1608 | 3104 |
| For slots without RMSI | Bits | 1864 | 1864 | 3624 |
| Number of Code Blocks per slot |  | 1 | 1 | 1 |
| Binary Channel Bits Per slot |  |  |  |  |
| For slots with RMSI Note 4 | Bits | 5184 | 5184 | 10368 |
| For slots without RMSI Note 6 | Bits | 6048 | 6048 | 12096 |
| NOTE 1: Allocated in resource blocks which do not overlap with the resource blocks allocated for SS/PBCH block. NOTE 2: Void NOTE 3: If necessary, the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in 3GPP TS 38.213 [8]. NOTE 4: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 2. NOTE 5: PDSCH is not scheduled in slots containing SSB according to the SSB configuration used in the test. SSB configurations are defined in clause A.3.10. NOTE 6: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditionalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1. NOTE 7: Allocated in the same resource blocks as the CORESET. Note 8: When DRX is configured, PDSCH is scheduled only while drx-onDurationTimer is running, unless otherwise specified in the test case. |  |  |  |  |

## A.1.2	CORESET for RMSI scheduling

### A.1.2.1	FDD

Table A.1.2.1-1: RMSI CORESET Reference Measurement Channels for SCS = 15 kHz for FDD

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | CR.1.1 FDD |
| Channel bandwidth | MHz | Defined in test case |
| Subcarrier spacing for RMSI CORESET | kHz | 15 |
| Allocated resource blocks for RMSI CORESETNote 7 |  | 24 |
| Subcarrier spacing for SSB | kHz | 15 |
| SSB and RMSI CORESET multiplexing configuration |  | Pattern 1 |
| Offset between SSB and RMSI CORESET Note 3, 7 | RB | 0 (Note 8) |
| Configuration of PDCCH monitoring occasions for RMSI CORESET Note 4 |  | Index 4 |
| Number of transmitter antennas |  | 1 |
| Duration of RMSI CORESETNote 7 | symbols | 2 |
| DCI Format Note 1 |  | Note 2 |
| Aggregation level | CCE | 8 |
| DMRS precoder granularity |  | 6 |
| REG bundle size |  | 6 |
| Mapping from REG to CCE |  | Distributed |
| Cell ID |  | Note 5 |
| Payload (without CRC) | Bits | Note 6 |
| NOTE 1: DCI formats are defined in TS 38.212 [31]. NOTE 2: DCI format shall depend upon the test configuration. NOTE 3: The offset is defined with respect to the subcarrier spacing of the CORESET from the smallest RB index of RMSI CORESET to the smallest RB index of the common RB overlapping with the first RB of the SS/PBCH block. NOTE 4: The configuration of PDCCH monitoring occasions for RMSI CORESET is defined in Table 13-11 in TS 38.213 [8]. NOTE 5: Cell ID shall depend upon the test configuration. NOTE 6: Payload size shall depend upon the test configuration. NOTE 7: The configuration of set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space corresponds to index 0 in Table 13-1 in TS 38.213 [8]. NOTE 8: Other values can be used to align with GSCN as long as SSB does not overlap the RMC |  |  |

### A.1.2.2	TDD

Table A.1.2.2-1: RMSI CORESET Reference Measurement Channels for SCS = 15 kHz for TDD

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | CR.1.1 TDD |
| Channel bandwidth | MHz | Defined in test case |
| Subcarrier spacing for RMSI CORESET | kHz | 15 |
| Allocated resource blocks for RMSI CORESET Note 7 |  | 24 |
| Index of transmitted SSB within an SS-Burst |  | #0 |
| SSB and RMSI CORESET multiplexing configuration |  | Pattern 1 |
| Offset between SSB and RMSI CORESET Note 3, 7 | RB | 0 (Note 8) |
| Configuration of PDCCH monitoring occasions for RMSI CORESET Note 4 |  | Index 4 |
| Number of transmitter antennas |  | 1 |
| Duration of RMSI CORESET Note 7 | symbols | 2 |
| DCI Format Note 1 |  | Note 2 |
| Aggregation level | CCE | 8 |
| DMRS precoder granularity |  | 6 |
| REG bundle size |  | 6 |
| Mapping from REG to CCE |  | Distributed |
| Cell ID |  | Note 5 |
| Payload (without CRC) | Bits | Note 6 |
| NOTE 1: DCI formats are defined in TS 38.212 [31]. NOTE 2: DCI format shall depend upon the test configuration. NOTE 3: The offset is defined with respect to the subcarrier spacing of the CORESET from the smallest RB index of RMSI CORESET to the smallest RB index of the common RB overlapping with the first RB of the SS/PBCH block. NOTE 4: The configuration of PDCCH monitoring occasions for RMSI CORESET is defined in Table 13-11 in TS 38.213 [8]. NOTE 5: Cell ID shall depend upon the test configuration. NOTE 6: Payload size shall depend upon the test configuration. NOTE 7: The configuration of set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space corresponds to index 0 in Table 13-1 in TS 38.213 [8]. NOTE 8: Other values can be used to align with GSCN as long as SSB does not overlap the RMC |  |  |

Table A.1.2.2-2: RMSI CORESET Reference Measurement Channels for SCS = 30 kHz for TDD

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | CR.2.1 TDD |
| Channel bandwidth | MHz | Defined in test case |
| Subcarrier spacing for RMSI CORESET | kHz | 30 |
| Allocated resource blocks for RMSI CORESET Note 7 |  | 24 |
| Index of transmitted SSB within an SS-Burst |  | #0 |
| SSB and RMSI CORESET multiplexing configuration |  | Pattern 1 |
| Offset between SSB and RMSI CORESET Note 3, 7 | RB | 0 (Note 8) |
| Configuration of PDCCH monitoring occasions for RMSI CORESET Note 4 |  | Index 4 |
| Number of transmitter antennas |  | 1 |
| Duration of RMSI CORESET Note 7 | symbols | 2 |
| DCI Format Note 1 |  | Note 2 |
| Aggregation level | CCE | 8 |
| DMRS precoder granularity |  | 6 |
| REG bundle size |  | 6 |
| Mapping from REG to CCE |  | Distributed |
| Cell ID |  | Note 5 |
| Payload (without CRC) | Bits | Note 6 |
| NOTE 1: DCI formats are defined in TS 38.212 [31]. NOTE 2: DCI format shall depend upon the test configuration. NOTE 3: The offset is defined with respect to the subcarrier spacing of the CORESET from the smallest RB index of RMSI CORESET to the smallest RB index of the common RB overlapping with the first RB of the SS/PBCH block. NOTE 4: The configuration of PDCCH monitoring occasions for RMSI CORESET is defined in Table 13-11 in TS 38.213 [8]. NOTE 5: Cell ID shall depend upon the test configuration. NOTE 6: Payload size shall depend upon the test configuration. NOTE 7: The configuration of set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space corresponds to index 0 in Table 13-6 in TS 38.213 [8]. NOTE 8: Other values can be used to align with GSCN as long as SSB does not overlap the RMC |  |  |

Table A.1.2.2-3: RMSI CORESET Reference Measurement Channels for SCS = 120 kHz for TDD

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | CR.3.1 TDD | CR.3.2 TDD |
| Channel bandwidth | MHz | 100 | 100 |
| Subcarrier spacing for RMSI CORESET | kHz | 120 | 120 |
| Allocated resource blocks for RMSI CORESET |  | 24Note 7 | 48Note 9 |
| Subcarrier spacing for SSB | kHz | 120 | 120 |
| Index of transmitted SSB within an SS-Burst |  | #0 | #0 |
| SSB and RMSI CORESET multiplexing configuration |  | Pattern 1Note 7 | Pattern 1Note 9 |
| Offset between SSB and RMSI CORESET Note 3 | RB | 0 (Note 8) Note 7 | 0 (Note 8)Note 9 |
| Configuration of PDCCH monitoring occasions for RMSI CORESET Note 4 |  | Index 4 | Index 4 |
| Number of transmitter antennas |  | 1 | 1 |
| Duration of RMSI CORESET | symbols | 2Note 7 | 2Note 9 |
| DCI Format Note 1 |  | Note 2 | Note 2 |
| Aggregation level | CCE | 8 | 8 |
| DMRS precoder granularity |  | 6 | 6 |
| REG bundle size |  | 6 | 6 |
| Mapping from REG to CCE |  | Distributed | Distributed |
| Cell ID |  | Note 5 | Note 5 |
| Payload (without CRC) | Bits | Note 6 | Note 6 |
| NOTE 1: DCI formats are defined in TS 38.212 [31]. NOTE 2: DCI format shall depend upon the test configuration. NOTE 3: The offset is defined with respect to the subcarrier spacing of the CORESET from the smallest RB index of RMSI CORESET to the smallest RB index of the common RB overlapping with the first RB of the SS/PBCH block. NOTE 4: The configuration of PDCCH monitoring occasions for RMSI CORESET is defined in Table 13-11 in TS 38.213 [8]. NOTE 5: Cell ID shall depend upon the test configuration. NOTE 6: Payload size shall depend upon the test configuration. NOTE 7: The configuration of set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space corresponds to index 0 in Table 13-8 in TS 38.213 [8]. NOTE 8: Other values can be used to align with GSCN as long as SSB does not overlap the RMC NOTE 9: The configuration of set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space corresponds to index 2 in Table 13-10 in TS 38.213 [8]. |  |  |  |

## A.1.3	CORESET for RMC scheduling

### A.1.3.1	FDD

Table A.1.3.1-1: Control Channel RMC for SCS = 15 kHz for FDD

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.1.1 FDD | CCR.1.2 FDD | CCR.1.3 FDD | CCR.1.4 FDD |
| Channel bandwidth | MHz | Defined in test case | Defined in test case | Defined in test case | Defined in test case |
| Subcarrier spacing for RMSI CORESET | kHz | 15 | 15 | 15 | 15 |
| Allocated resource blocks for CORESET |  | 24 | 18 | 24 | 18 |
| Number of transmitter antenna |  | 1 | 1 | 1 | 1 |
| Duration of CORESET | symbols | 2 | 2 | 2 | 2 |
| REG bundle size |  | 6 | 6 | 6 | 6 |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size |
| CCE to REG mapping |  | Interleaved | Interleaved | Interleaved | Interleaved |
| Interleave n_shift |  | 0 | 0 | 0 | 0 |
| Interleave size |  | 2 | 2 | 2 | 2 |
| Beamforming Pre-Coder |  | N/A | N/A | N/A | N/A |
| Aggregation level | CCE | 4 | 2 | 8 | 4 |
| DCI formats |  | Note 1 | Note 1 | Note 1 | Note 1 |
| Payload size (without CRC) | bits | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: DCI format shall depend upon the test configuration. NOTE 2: Payload size shall depend upon the test configuration NOTE 3: Allocated in the resource blocks where the associated RMC is scheduled. |  |  |  |  |  |

### A.1.3.2	TDD

Table A.1.3.2-1: Control Channel RMC for SCS = 15 kHz for TDD

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.1.1 TDD | CCR.1.2 TDD | CCR.1.3 TDD | CCR.1.4 TDD |
| Channel bandwidth | MHz | Defined in test case | Defined in test case | Defined in test case | Defined in test case |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 |
| Allocated resource blocks for CORESET Note3 |  | 24 | 18 | 24 | 18 |
| Number of transmitter antennas |  | 1 | 1 | 1 | 1 |
| Duration of CORESET | symbols | 2 | 2 | 2 | 2 |
| REG bundle size |  | 6 | 6 | 6 | 6 |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size |
| CCE to REG mapping |  | Interleaved | Interleaved | Interleaved | Interleaved |
| Interleave n_shift |  | 0 | 0 | 0 | 0 |
| Interleave size |  | 2 | 2 | 2 | 2 |
| Beamforming Pre-Coder |  | N/A | N/A | N/A | N/A |
| Aggregation level | CCE | 4 | 2 | 8 | 4 |
| DCI formats |  | Note 1 | Note 1 | Note 1 | Note 1 |
| Payload size (without CRC) | bits | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: DCI format shall depend upon the test configuration. NOTE 2: Payload size shall depend upon the test configuration NOTE 3: Allocated in the resource blocks where the associated RMC is scheduled. |  |  |  |  |  |

Table A.1.3.2-2: Control Channel RMC for SCS = 30 kHz for TDD

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.2.1 TDD | CCR.2.2 TDD | CCR.2.3 TDD | CCR.2.4 TDD |
| Channel bandwidth | MHz | 40 | 40 | 40 | 40 |
| Subcarrier spacing for RMSI CORESET | kHz | 30 | 30 | 30 | 30 |
| Allocated resource blocks for CORESET3 |  | 24 | 24 | 18 | 18 |
| Number of transmitter antenna |  | 1 | 1 | 1 | 1 |
| Duration of CORESET | symbols | 2 | 2 | 2 | 2 |
| REG bundle size |  | 6 | 6 | 6 | 6 |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size |
| CCE to REG mapping |  | Interleaved | Interleaved | Interleaved | Interleaved |
| Interleave n_shift |  | 0 | 0 | 0 | 0 |
| Interleave size |  | 2 | 2 | 2 | 2 |
| Beamforming Pre-Coder |  | N/A | N/A | N/A | N/A |
| Aggregation level | CCE | 4 | 8 | 4 | 2 |
| DCI formats |  | Note 1 | Note 1 | Note 1 | Note 1 |
| Payload size (without CRC) | bits | Note 2 | Note 2 | Note 2 | Note 2 |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration. Note 3: Allocated in the same resource blocks where the associated RMC is scheduled. |  |  |  |  |  |

Table A.1.3.2-3: Control Channel RMC for SCS = 120 kHz for TDD

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.3.1 TDD | CCR.3.2 TDD | CCR.3.3 TDD | CCR.3.4 TDD | CCR.3.5 TDD | CCR.3.6 TDD | CCR.3.7 TDD |
| Channel bandwidth | MHz | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| Subcarrier spacing for RMSI CORESET | kHz | 120 | 120 | 120 | 120 | 120 | 120 | 120 |
| Allocated resource blocks for CORESETNote 3 |  | 24 | 24 | 24 | 24 | 24 | 24 | 48 |
| Number of transmitter antenna |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| monitoringSlotPeriodicityAndOffsetNote 4 |  | sl160 0 | sl160 0 | sl160 80 | sl160 0 | sl160 0 | sl160 80 | sl160 0 |
| monitoringSymbolsWithinSlot |  | 1100000 0000000 | 0011000 0000000 | 1100000 0000000 | 1100000 0000000 | 0011000 0000000 | 1100000 0000000 | 1100000 0000000 |
| Duration of CORESET | symbols | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| REG bundle size |  | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size |
| CCE to REG mapping |  | Interleaved | Interleaved | Interleaved | Interleaved | Interleaved | Interleaved | Interleaved |
| Interleave n_shift |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Interleave size |  | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| Beamforming Pre-Coder |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Aggregation level | CCE | 4 | 4 | 4 | 8 | 8 | 8 | 4 |
| DCI formats |  | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 |
| Payload size (without CRC) | bits | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: DCI format shall depend upon the test configuration. NOTE 2: Payload size shall depend upon the test configuration NOTE 3: Allocated in the same resource blocks where the associated PDSCH RMC is scheduled. NOTE 4: monitoringSlotPeriodicityAndOffet can be set to other values depending on test purpose of respective test cases, as needed, e.g. monitoringSlotPeriodicityAndOffet is set to “sl1 0” if it is specifically stated that cell(s) configured with one of the control channel RMCs above shall transmit PDCCHs continuously. |  |  |  |  |  |  |  |  |

## A.1.4	CSI-RS

### A.1.4.1	FDD

Table A.1.4.1-1: CSI-RS Reference Measurement Channels for SCS = 15 kHz for FDD

|  | CSI-RS.1.1 FDD | CSI-RS.1.2 FDD | CSI-RS.1.3 FDD | CSI-RS.1.4 FDD |
| --- | --- | --- | --- | --- |
| Resource Type | periodic | periodic | aperiodic | aperiodic |
| Resource Set Config |  |  |  |  |
| nzp-CSI-ResourceSetId | 0 | 0 | 0 | 0 |
| repetition | N/A | off | off | on |
| aperiodicTriggeringOffset | N/A | N/A | 4 | 4 |
| trs-Info | N/A | N/A | N/A | N/A |
| Resource Config |  |  |  |  |
| nzp-CSI-RS-ResourceId | 0 for resource #0 | 0 for resource #0 | 0 for resource #0 | 0 for resource #0 |
|  |  |  |  | 1 for resource #1 |
|  |  |  |  | 2 for resource #2 |
|  |  |  |  | 3 for resource #3 |
|  |  | 1 for resource #1 | 1 for resource #1 | 4 for resource #4 |
|  |  |  |  | 5 for resource #5 |
|  |  |  |  | 6 for resource #6 |
|  |  |  |  | 7 for resource #7 |
| powerControlOffset | 0 | 0 | 0 | 0 |
| powerControlOffsetSS | db0 | db0 | db0 | db0 |
| scramblingID | 0 | 0 | 0 | 0 |
| Period (slots) | slot5 | slot10 | N/A | N/A |
| Offset | 1 | 1 | N/A | N/A |
| qcl-InfoPeriodicCSI-RS | TCI.State.0 | TCI.State.0 | N/A | N/A |
|  |  | TCI.State.1 |  |  |
| frequencyDomainAllocation | 000001 | 0001 | 0001 | 0001 |
| nrofPorts | 2 | 1 | 1 | 1 |
| firstOFDMSymbolInTimeDomain | 4 for resource #0 | 6 for resource #0 | 6 for resource #0 | 0 for resource #0 |
|  |  |  |  | 1 for resource #1 |
|  |  |  |  | 2 for resource #2 |
|  |  |  |  | 3 for resource #3 |
|  |  | 10 for resource #1 | 10 for resource #1 | 4 for resource #4 |
|  |  |  |  | 5 for resource #5 |
|  |  |  |  | 6 for resource #6 |
|  |  |  |  | 7 for resource #7 |
| cdm-Type | FD-CDM2 | noCDM | noCDM | noCDM |
| density | 1 | 3 | 3 | 3 |
| startingRB | 0 | 0 | 0 | 0 |
| nrofRBs | 276 | 276 | 276 | 276 |

Table A.1.4.1-1A: CSI-RS Reference Measurement Channels for SCS=15 kHz for FDD

|  | CSI-RS.1.1A FDD | CSI-RS.1.2A FDD | CSI-RS.1.3A FDD |
| --- | --- | --- | --- |
| Resource Type | periodic | aperiodic | periodic |
| Resource Set Config |  |  |  |
| nzp-CSI-ResourceSetId | 1 | 1 | 1 |
| repetition | off | off | off |
| aperiodicTriggeringOffset | n.a. | 6 | n.a. |
| trs-Info | n.a. | n.a. | n.a. |
| Resource Config |  |  |  |
| nzp-CSI-RS-ResourceId | 12 for resource #0 | 22 for resource #0 | 14 for resource #0 |
|  | 13 for resource #1 | 23 for resource #1 | 15 for resource #1 |
| powerControlOffset | 0 | 0 | 0 |
| powerControlOffsetSS | db0 | db0 | db0 |
| scramblingID | 0 | 0 | 0 |
| Period (slots) | slot20 | n.a. | slot10 |
| Offset | 1 | n.a. | 2 |
| qcl-InfoPeriodicCSI-RS | n.a. | n.a. | n.a. |
| frequencyDomainAllocation | 0001 | 0001 | 0001 |
| nrofPorts | 1 | 1 | 1 |
| firstOFDMSymbolInTimeDomain | 6 for resource #0 | 7 for resource #0 | 6 for resource #0 |
|  | 10 for resource #1 | 11 for resource #1 | 10 for resource #1 |
| cdm-Type | noCDM | noCDM | noCDM |
| density | 3 | 3 | 3 |
| startingRB | 0 | 0 | 0 |
| nrofRBs | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) |
| Note 1: If the configured value of PRBs is larger than the width of the corresponding BWP relevant for the test case, the Test Equipment shall implement CSI-RS only in the width of that BWP. |  |  |  |

### A.1.4.2	TDD

Table A.1.4.2-1: CSI-RS Reference Measurement Channels for SCS = 15 kHz for TDD

|  | CSI-RS.1.1 TDD | CSI-RS.1.2 TDD | CSI-RS.1.3 TDD | CSI-RS.1.4 TDD |
| --- | --- | --- | --- | --- |
| Resource Type | periodic | periodic | aperiodic | aperiodic |
| Resource Set Config |  |  |  |  |
| nzp-CSI-ResourceSetId | 0 | 0 | 0 | 0 |
| repetition | N/A | off | off | on |
| aperiodicTriggeringOffset | N/A | N/A | 4 | 4 |
| trs-Info | N/A | N/A | N/A | N/A |
| Resource Config |  |  |  |  |
| nzp-CSI-RS-ResourceId | 0 for resource #0 | 0 for resource #0 | 0 for resource #0 | 0 for resource #0 |
|  |  |  |  | 1 for resource #1 |
|  |  |  |  | 2 for resource #2 |
|  |  |  |  | 3 for resource #3 |
|  |  | 1 for resource #1 | 1 for resource #1 | 4 for resource #4 |
|  |  |  |  | 5 for resource #5 |
|  |  |  |  | 6 for resource #6 |
|  |  |  |  | 7 for resource #7 |
| powerControlOffset | 0 | 0 | 0 | 0 |
| powerControlOffsetSS | db0 | db0 | db0 | db0 |
| scramblingID | 0 | 0 | 0 | 0 |
| Period (slots) | slot5 | slot10 | N/A | N/A |
| Offset | 1 | 1 | N/A | N/A |
| qcl-InfoPeriodicCSI-RS | TCI.State.0 | TCI.State.0 | N/A | N/A |
|  |  | TCI.State.1 |  |  |
| frequencyDomainAllocation | 000001 | 0001 | 0001 | 0001 |
| nrofPorts | 2 | 1 | 1 | 1 |
| firstOFDMSymbolInTimeDomain | 4 for resource #0 | 6 for resource #0 | 6 for resource #0 | 0 for resource #0 |
|  |  |  |  | 1 for resource #1 |
|  |  |  |  | 2 for resource #2 |
|  |  |  |  | 3 for resource #3 |
|  |  | 10 for resource #1 | 10 for resource #1 | 4 for resource #4 |
|  |  |  |  | 5 for resource #5 |
|  |  |  |  | 6 for resource #6 |
|  |  |  |  | 7 for resource #7 |
| cdm-Type | FD-CDM2 | noCDM | noCDM | noCDM |
| density | 1 | 3 | 3 | 3 |
| startingRB | 0 | 0 | 0 | 0 |
| nrofRBs | 276 | 276 | 276 | 276 |

Table A.1.4.2-1A: CSI-RS Reference Measurement Channels for SCS=15 kHz for TDD

|  | CSI-RS.1.1A TDD | CSI-RS.1.2A TDD | CSI-RS.1.3A TDD |
| --- | --- | --- | --- |
| Resource Type | periodic | aperiodic | periodic |
| Resource Set Config |  |  |  |
| nzp-CSI-ResourceSetId | 1 | 1 | 1 |
| repetition | off | off | off |
| aperiodicTriggeringOffset | n.a. | 6 | n.a. |
| trs-Info | n.a. | n.a. | n.a. |
| Resource Config |  |  |  |
| nzp-CSI-RS-ResourceId | 12 for resource #0 | 22 for resource #0 | 14 for resource #0 |
|  | 13 for resource #1 | 23 for resource #1 | 15 for resource #1 |
| powerControlOffset | 0 | 0 | 0 |
| powerControlOffsetSS | db0 | db0 | db0 |
| scramblingID | 0 | 0 | 0 |
| Period (slots) | slot20 | n.a. | slot10 |
| Offset | 1 | n.a. | 2 |
| qcl-InfoPeriodicCSI-RS | n.a. | n.a. | n.a. |
| frequencyDomainAllocation | 0001 | 0001 | 0001 |
| nrofPorts | 1 | 1 | 1 |
| firstOFDMSymbolInTimeDomain | 6 for resource #0 | 7 for resource #0 | 6 for resource #0 |
|  | 10 for resource #1 | 11 for resource #1 | 10 for resource #1 |
| cdm-Type | noCDM | noCDM | noCDM |
| density | 3 | 3 | 3 |
| startingRB | 0 | 0 | 0 |
| nrofRBs | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) |
| Note 1: If the configured value of PRBs is larger than the width of the corresponding BWP relevant for the test case, the Test Equipment shall implement CSI-RS only in the width of that BWP. |  |  |  |

Table A.1.4.2-2: CSI-RS Reference Measurement Channels for SCS = 30 kHz for TDD

|  | CSI-RS.2.1 TDD | CSI-RS.2.2 TDD | CSI-RS.2.3 TDD | CSI-RS.2.4 TDD |
| --- | --- | --- | --- | --- |
| Resource Type | periodic | periodic | aperiodic | aperiodic |
| Resource Set Config |  |  |  |  |
| nzp-CSI-ResourceSetId | 0 | 0 | 0 | 0 |
| repetition | N/A | off | off | on |
| aperiodicTriggeringOffset | N/A | N/A | 4 | 4 |
| trs-Info | N/A | N/A | N/A | N/A |
| Resource Config |  |  |  |  |
| nzp-CSI-RS-ResourceId | 0 for resource #0 | 0 for resource #0 | 0 for resource #0 | 0 for resource #0 |
|  |  |  |  | 1 for resource #1 |
|  |  |  |  | 2 for resource #2 |
|  |  |  |  | 3 for resource #3 |
|  |  | 1 for resource #1 | 1 for resource #1 | 4 for resource #4 |
|  |  |  |  | 5 for resource #5 |
|  |  |  |  | 6 for resource #6 |
|  |  |  |  | 7 for resource #7 |
| powerControlOffset | 0 | 0 | 0 | 0 |
| powerControlOffsetSS | db0 | db0 | db0 | db0 |
| scramblingID | 0 | 0 | 0 | 0 |
| Period (slots) | slot10 | slot20 | N/A | N/A |
| Offset | 2 | 2 | N/A | N/A |
| qcl-InfoPeriodicCSI-RS | TCI.State.0 | TCI.State.0 | N/A | N/A |
|  |  | TCI.State.1 |  |  |
| frequencyDomainAllocation | 000001 | 0001 | 0001 | 0001 |
| nrofPorts | 2 | 1 | 1 | 1 |
| firstOFDMSymbolInTimeDomain | 5 for resource #0 | 6 for resource #0 | 6 for resource #0 | 0 for resource #0 |
|  |  |  |  | 1 for resource #1 |
|  |  |  |  | 2 for resource #2 |
|  |  |  |  | 3 for resource #3 |
|  |  | 10 for resource #1 | 10 for resource #1 | 4 for resource #4 |
|  |  |  |  | 5 for resource #5 |
|  |  |  |  | 6 for resource #6 |
|  |  |  |  | 7 for resource #7 |
| cdm-Type | FD-CDM2 | noCDM | noCDM | noCDM |
| density | 1 | 3 | 3 | 3 |
| startingRB | 0 | 0 | 0 | 0 |
| nrofRBs | 276 | 276 | 276 | 276 |

Table A.1.4.2-2A: CSI-RS Reference Measurement Channels for SCS=30 kHz for TDD

|  | CSI-RS.2.1A TDD | CSI-RS.2.2A TDD | CSI-RS.2.3A TDD |
| --- | --- | --- | --- |
| Resource Type | periodic | aperiodic | periodic |
| Resource Set Config |  |  |  |
| nzp-CSI-ResourceSetId | 1 | 1 | 1 |
| repetition | off | off | off |
| aperiodicTriggeringOffset | n.a. | 6 | n.a. |
| trs-Info | n.a. | n.a. | n.a. |
| Resource Config |  |  |  |
| nzp-CSI-RS-ResourceId | 12 for resource #0 | 22 for resource #0 | 14 for resource #0 |
|  | 13 for resource #1 | 23 for resource #1 | 15 for resource #1 |
| powerControlOffset | 0 | 0 | 0 |
| powerControlOffsetSS | db0 | db0 | db0 |
| scramblingID | 0 | 0 | 0 |
| Period (slots) | slot40 | n.a. | slot20 |
| Offset | 2 | n.a. | 4 |
| qcl-InfoPeriodicCSI-RS | n.a. | n.a. | n.a. |
| frequencyDomainAllocation | 0001 | 0001 | 0001 |
| nrofPorts | 1 | 1 | 1 |
| firstOFDMSymbolInTimeDomain | 6 for resource #0 | 7 for resource #0 | 6 for resource #0 |
|  | 10 for resource #1 | 11 for resource #1 | 10 for resource #1 |
| cdm-Type | noCDM | noCDM | noCDM |
| density | 3 | 3 | 3 |
| startingRB | 0 | 0 | 0 |
| nrofRBs | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) |
| Note 1: If the configured value of PRBs is larger than the width of the corresponding BWP relevant for the test case, the Test Equipment shall implement CSI-RS only in the width of that BWP. |  |  |  |

Table A.1.4.2-3: CSI-RS Reference Measurement Channels for SCS = 120 kHz for TDD

|  | CSI-RS.3.1 TDD | CSI-RS.3.2 TDD | CSI-RS.3.3 TDD | CSI-RS.3.4 TDD |
| --- | --- | --- | --- | --- |
| Resource Type | periodic | periodic | aperiodic | aperiodic |
| Resource Set Config |  |  |  |  |
| nzp-CSI-ResourceSetId | 0 | 0 | 0 | 0 |
| repetition | N/A | off | off | on |
| aperiodicTriggeringOffset | N/A | N/A | 4 | 4 |
| trs-Info | N/A | N/A | N/A | N/A |
| Resource Config |  |  |  |  |
| nzp-CSI-RS-ResourceId | 0 for resource #0 | 0 for resource #0 | 0 for resource #0 | 0 for resource #0 |
|  |  |  |  | 1 for resource #1 |
|  |  |  |  | 2 for resource #2 |
|  |  |  |  | 3 for resource #3 |
|  |  | 1 for resource #1 | 1 for resource #1 | 4 for resource #4 |
|  |  |  |  | 5 for resource #5 |
|  |  |  |  | 6 for resource #6 |
|  |  |  |  | 7 for resource #7 |
| powerControlOffset | 0 | 0 | 0 | 0 |
| powerControlOffsetSS | db0 | db0 | db0 | db0 |
| scramblingID | 0 | 0 | 0 | 0 |
| Period (slots) | slot40 | slot80 | N/A | N/A |
| Offset | 8 | 8 | N/A | N/A |
| qcl-InfoPeriodicCSI-RS | TCI.State.0 | TCI.State.0 | N/A | N/A |
|  |  | TCI.State.1 |  |  |
| frequencyDomainAllocation | 000001 | 0001 | 0001 | 0001 |
| nrofPorts | 2 | 1 | 1 | 1 |
| firstOFDMSymbolInTimeDomain | 5 for resource #0 | 6 for resource #0 | 6 for resource #0 | 0 for resource #0 |
|  |  |  |  | 1 for resource #1 |
|  |  |  |  | 2 for resource #2 |
|  |  |  |  | 3 for resource #3 |
|  |  | 10 for resource #1 | 10 for resource #1 | 4 for resource #4 |
|  |  |  |  | 5 for resource #5 |
|  |  |  |  | 6 for resource #6 |
|  |  |  |  | 7 for resource #7 |
| cdm-Type | FD-CDM2 | noCDM | noCDM | noCDM |
| density | 1 | 3 | 3 | 3 |
| startingRB | 0 | 0 | 0 | 0 |
| nrofRBs | 276 | 276 | 276 | 276 |

Table A.1.4.2-3A: CSI-RS Reference Measurement Channels for SCS=120 kHz for TDD

|  | CSI-RS.3.1A TDD | CSI-RS.3.2A TDD | CSI-RS.3.3A TDD |
| --- | --- | --- | --- |
| Resource Type | periodic | aperiodic | periodic |
| Resource Set Config |  |  |  |
| nzp-CSI-ResourceSetId | 1 | 1 | 1 |
| repetition | off | off | off |
| aperiodicTriggeringOffset | n.a. | 6 | n.a. |
| trs-Info | n.a. | n.a. | n.a. |
| Resource Config |  |  |  |
| nzp-CSI-RS-ResourceId | 12 for resource #0 | 22 for resource #0 | 14 for resource #0 |
|  | 13 for resource #1 | 23 for resource #1 | 15 for resource #1 |
| powerControlOffset | 0 | 0 | 0 |
| powerControlOffsetSS | db0 | db0 | db0 |
| scramblingID | 0 | 0 | 0 |
| Period (slots) | slot160 | n.a. | slot80 |
| Offset | 8 | n.a. | 16 |
| qcl-InfoPeriodicCSI-RS | n.a. | n.a. | n.a. |
| frequencyDomainAllocation | 0001 | 0001 | 0001 |
| nrofPorts | 1 | 1 | 1 |
| firstOFDMSymbolInTimeDomain | 6 for resource #0 | 7 for resource #0 | 6 for resource #0 |
|  | 10 for resource #1 | 11 for resource #1 | 10 for resource #1 |
| cdm-Type | noCDM | noCDM | noCDM |
| density | 3 | 3 | 3 |
| startingRB | 0 | 0 | 0 |
| nrofRBs | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) |
| Note 1: If the configured value of PRBs is larger than the width of the corresponding BWP relevant for the test case, the Test Equipment shall implement CSI-RS only in the width of that BWP. |  |  |  |

## A.1.4A	CSI-RS for tracking

### A.1.4A.1	FR1

#### A.1.4A.1.1	FDD

Table A.1.4A.1.1-1: CSI-RS for tracking FDD

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | TRS.1.1 FDD | TRS.1.2 FDD |
| Bandwidth |  | BW of Active BWP1 | BW of Active BWP1 |
| SCS | kHz | 15 | 30 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 20 for CSI-RS resource 1,2,3,4 | 40 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 | 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0 | 0 |
| TCI state |  | TCI.State.0 | TCI.State.0 |
| NOTE 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases |  |  |  |

#### A.1.4A.1.2	TDD

Table A.1.4A.1.2-1: CSI-RS for TDD

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | TRS.1.1 TDD | TRS.1.2 TDD |
| Bandwidth |  | BW of Active BWP1 | BW of Active BWP1 |
| SCS | kHz | 15 | 30 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 20 for CSI-RS resource 1,2,3,4 | 40 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 | 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0 | 0 |
| TCI state |  | TCI.State.0 | TCI.State.0 |
| NOTE 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases |  |  |  |

### A.1.4A.2	FR2

#### A.1.4A.2.1	TDD

Table A.1.4A.2.1-1: CSI-RS for tracking for TDD FR2

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | TRS.2.1 TDD | TRS.2.2 TDD |
| Bandwidth |  | BW of Active BWPNote 1, 2 | BW of Active BWPNote 1,2 |
| SCS | kHz | 120 | 120 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 1 for CSI-RS resource 1 and 3 l0 = 5 for CSI-RS resource 2 and 4 | l0 = 2 for CSI-RS resource 1 and 3 l0 = 6 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 80 for CSI-RS resource 1,2,3,4 | 80 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 40 for CSI-RS resource 1 and 2 41 for CSI-RS resource 3 and 4 | 40 for CSI-RS resource 1 and 2 41 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0 | 0 |
| TCI state |  | TCI.State.0 | TCI.State.1 |
| NOTE 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases NOTE 2: If active BWP is larger than 52RBs, BW of TRS is configured as 52RBs. Otherwise, same as active BWP size. |  |  |  |

## A.1.4B	CSI-IM configurations

### A.1.4B.1	FDD

Table A.1.4B.1-1: CSI-IM Reference Measurement Channels for SCS=15kHz

|  | CSI-IM.1.1 FDD | CSI-IM.1.2 FDD | CSI-IM.1.3 FDD |
| --- | --- | --- | --- |
| Resource Type | periodic | aperiodic | periodic |
| Resource Set Config |  |  |  |
| csi-IM-ResourceSetId | 0 | 0 | 0 |
| Resource Config |  |  |  |
| csi-IM-ResourceId | 0 for resource #0 | 10 for resource #0 | 2 for resource #0 |
|  | 1 for resource #1 | 11 for resource #1 | 3 for resource #1 |
| csi-IM-ResourceElementPattern | pattern1 | pattern1 | pattern1 |
| subcarrierLocation-p1 | s0 | s0 | s0 |
| symbolLocation-p1 | 6 for resource #0 | 7 for resource #0 | 6 for resource #0 |
|  | 10 for resource #1 | 11 for resource #1 | 10 for resource #1 |
| Period (slots) | slot20 | n.a. | slot10 |
| Offset | 1 | n.a. | 2 |
| startingRB | 0 | 0 | 0 |
| nrofRBs | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) |
| Note 1: If the configured value of PRBs is larger than the width of the corresponding BWP relevant for the test case, the test Equipment shall implement CSI-RS only in the width of that BWP. |  |  |  |

### A.1.4B.2	TDD

Table A.1.4B.2-1: CSI-IM Reference Measurement Channels for SCS=15kHz

|  | CSI-IM.1.1 TDD | CSI-IM.1.2 TDD | CSI-IM.1.3 TDD |
| --- | --- | --- | --- |
| Resource Type | periodic | aperiodic | periodic |
| Resource Set Config |  |  |  |
| csi-IM-ResourceSetId | 0 | 0 | 0 |
| Resource Config |  |  |  |
| csi-IM-ResourceId | 0 for resource #0 | 10 for resource #0 | 2 for resource #0 |
|  | 1 for resource #1 | 11 for resource #1 | 3 for resource #1 |
| csi-IM-ResourceElementPattern | pattern1 | pattern1 | pattern1 |
| subcarrierLocation-p1 | s0 | s0 | s0 |
| symbolLocation-p1 | 6 for resource #0 | 7 for resource #0 | 6 for resource #0 |
|  | 10 for resource #1 | 11 for resource #1 | 10 for resource #1 |
| Period (slots) | slot20 | n.a. | slot10 |
| Offset | 1 | n.a. | 2 |
| startingRB | 0 | 0 | 0 |
| nrofRBs | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) |
| Note 1: If the configured value of PRBs is larger than the width of the corresponding BWP relevant for the test case, the test Equipment shall implement CSI-RS only in the width of that BWP. |  |  |  |

Table A.1.4B.2-2: CSI-IM Reference Measurement Channels for SCS=30kHz

|  | CSI-IM.2.1 TDD | CSI-IM.2.2 TDD | CSI-IM.2.3 TDD |
| --- | --- | --- | --- |
| Resource Type | periodic | aperiodic | periodic |
| Resource Set Config |  |  |  |
| csi-IM-ResourceSetId | 0 | 0 | 0 |
| Resource Config |  |  |  |
| csi-IM-ResourceId | 0 for resource #0 | 10 for resource #0 | 2 for resource #0 |
|  | 1 for resource #1 | 11 for resource #1 | 3 for resource #1 |
| csi-IM-ResourceElementPattern | pattern1 | pattern1 | pattern1 |
| subcarrierLocation-p1 | s0 | s0 | s0 |
| symbolLocation-p1 | 6 for resource #0 | 7 for resource #0 | 6 for resource #0 |
|  | 10 for resource #1 | 11 for resource #1 | 10 for resource #1 |
| Period (slots) | slot40 | n.a. | slot40 |
| Offset | 2 | n.a. | 4 |
| startingRB | 0 | 0 | 0 |
| nrofRBs | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) |
| Note 1: If the configured value of PRBs is larger than the width of the corresponding BWP relevant for the test case, the test Equipment shall implement CSI-RS only in the width of that BWP. |  |  |  |

Table A.1.4B.2-3: CSI-RS Reference Measurement Channels for SCS=120kHz

|  | CSI-IM.3.1 TDD | CSI-IM.3.2 TDD | CSI-IM.3.3 TDD |
| --- | --- | --- | --- |
| Resource Type | periodic | aperiodic | periodic |
| Resource Set Config |  |  |  |
| csi-IM-ResourceSetId | 0 | 0 | 0 |
| Resource Config |  |  |  |
| csi-IM-ResourceId | 0 for resource #0 | 10 for resource #0 | 2 for resource #0 |
|  | 1 for resource #1 | 11 for resource #1 | 3 for resource #1 |
| csi-IM-ResourceElementPattern | pattern1 | pattern1 | pattern1 |
| subcarrierLocation-p1 | s0 | s0 | s0 |
| symbolLocation-p1 | 6 for resource #0 | 7 for resource #0 | 6 for resource #0 |
|  | 10 for resource #1 | 11 for resource #1 | 10 for resource #1 |
| Period (slots) | slot160 | n.a. | slot80 |
| Offset | 8 | n.a. | 16 |
| startingRB | 0 | 0 | 0 |
| nrofRBs | 276 (Note 1) | 276 (Note 1) | 276 (Note 1) |
| Note 1: If the configured value of PRBs is larger than the width of the corresponding BWP relevant for the test case, the test Equipment shall implement CSI-RS only in the width of that BWP. |  |  |  |

## A.1.5	TDD UL/DL configuration

Table A.1.5-1: TDD UL/DL configuration for SCS=15kHz

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | TDDConf.1.1 |  |  |
| referenceSubcarrierSpacing | kHz | 15 |  |  |
| TDD UL/DL pattern 1 Note 2 |  | ‘DSUU’ S=‘10DL:2GP:2UL’ |  |  |
| dl-UL-TransmissionPeriodicity | ms | 4 |  |  |
| nrofDownlinkSlots |  | 1 |  |  |
| nrofDownlinkSymbols |  | 10 |  |  |
| nrofUplinkSlot |  | 2 |  |  |
| nrofUplinkSymbols |  | 2 |  |  |
| TDD UL/DL pattern 2 Note 2 |  | ‘D’ |  |  |
| dl-UL-TransmissionPeriodicity | ms | 1 |  |  |
| nrofDownlinkSlots |  | 1 |  |  |
| nrofDownlinkSymbols |  | 0 |  |  |
| nrofUplinkSlot |  | 0 |  |  |
| nrofUplinkSymbols |  | 0 |  |  |
| NOTE 1: As specified in TS 38.213 [8] and TS 38.331 [13]. NOTE 2: For information |  |  |  |  |

Table A.1.5-2: TDD UL/DL configuration for SCS=30kHz

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | TDDConf.2.1 |  |  |
| referenceSubcarrierSpacing | kHz | 30 |  |  |
| TDD UL/DL pattern 1 Note 2 |  | ‘3D1S4U’ S=‘6DL:4GP:4UL’ |  |  |
| dl-UL-TransmissionPeriodicity | ms | 4 |  |  |
| nrofDownlinkSlots |  | 3 |  |  |
| nrofDownlinkSymbols |  | 6 |  |  |
| nrofUplinkSlot |  | 4 |  |  |
| nrofUplinkSymbols |  | 4 |  |  |
| TDD UL/DL pattern 2 Note 2 |  | ‘DD’ |  |  |
| dl-UL-TransmissionPeriodicity | ms | 1 |  |  |
| nrofDownlinkSlots |  | 2 |  |  |
| nrofDownlinkSymbols |  | 0 |  |  |
| nrofUplinkSlot |  | 0 |  |  |
| nrofUplinkSymbols |  | 0 |  |  |
| NOTE 1: As specified in TS 38.213 [8] and TS 38.331 [13]. NOTE 2: For information |  |  |  |  |

Table A.1.5-3: TDD UL/DL configuration for SCS=120kHz

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | TDDConf.3.1 |  |  |
| referenceSubcarrierSpacing | kHz | 120 |  |  |
| TDD UL/DL pattern 1 Note 2 |  | ‘DDDSU’ S=‘10DL:2GP:2UL’ |  |  |
| dl-UL-TransmissionPeriodicity | ms | 0.625 |  |  |
| nrofDownlinkSlots |  | 3 |  |  |
| nrofDownlinkSymbols |  | 10 |  |  |
| nrofUplinkSlot |  | 1 |  |  |
| nrofUplinkSymbols |  | 2 |  |  |
| TDD UL/DL pattern 2 Note 2 |  | Not configured |  |  |
| dl-UL-TransmissionPeriodicity | ms | Not configured |  |  |
| nrofDownlinkSlots |  | Not configured |  |  |
| nrofDownlinkSymbols |  | Not configured |  |  |
| nrofUplinkSlot |  | Not configured |  |  |
| nrofUplinkSymbols |  | Not configured |  |  |
| NOTE 1: As specified in TS 38.213 [8] and TS 38.331 [13]. NOTE 2: For information |  |  |  |  |

## A.1.6	PUSCH

This rule applies to NR cell(s), which the UE is connected to. The UE is in RRC_CONNECTED mode.

When signalling or data payloads are expected to be sent on the PUSCH, the UE may be provided in advance with PUSCH resources by the SS. For sake of simplicity, the PUSCH scheduling may also occur continuously over many consecutive subframes. These options shall not be used if:
1) stated otherwise in the test description, or 
2) the transmission of PUSCH and UL scheduling information affects the test purpose (e.g. DRX, PUCCH reception etc.)

# A.2	Reference OCGN configuration

## A.2.1 Generic OFDMA channel noise generator (OCNG)

The OCNGpattern is used in a test for modelling the allocation of unused resourced in the channel bandwidth to virtual UEs (UEs that are not under test). The OCNG pattern simulates both PDCCH and PDSCH transmissions to the virtual UEs.

Table A.2.1-1: OP.1: Generic OCNG pattern for all unused REs

| OCNG Parameters | Control Region | Data Region |
| --- | --- | --- |
| Resource allocation | Unused REs (Note 1) | Unused REs (Note 2) |
| Channel | PDCCH | PDSCH |
| Contents | Virtual UE IDs | Uncorrelated pseudo random QPSK modulated data |
| Antenna transmission scheme | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Subcarrier spacing | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Aggregation level | Same as used in PDCCH RMC | N/A |
| Code rate | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Transmit Power | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| CP length | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| NOTE 1: REs not used in the active CORESETs where PDCCH is scheduled for the UE under test. NOTE 2: REs not allocated to any physical channels, CORESET, SSB or any other reference signal within the channel bandwidth of the cell, confined to BWoccupied where specified in the test case. |  |  |

Table A.2.1-2: OP.2: Generic OCNG pattern for all unused REs for 2AoA setup

| OCNG Parameters | Control Region | Data Region |
| --- | --- | --- |
| Probe | Transmitting the serving beam |  |
| Resource allocation | Unused REs (Note 1) in the symbols where SSB/CSI-RS are not transmitted from both the serving beam probe and non-serving beam probe. | Unused REs (Note 2) in the symbols where SSB/CSI-RS are not transmitted from both the serving beam probe and non-serving beam probe. |
| Channel | PDCCH | PDSCH |
| Contents | Virtual UE IDs | Uncorrelated pseudo random QPSK modulated data |
| Antenna transmission scheme | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Subcarrier spacing | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Aggregation level | Same as used in PDCCH RMC | N/A |
| Code rate | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Transmit Power | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| CP length | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| NOTE 1: REs not used in the active CORESETs where PDCCH is scheduled for the UE under test. NOTE 2: REs not allocated to any physical channels, CORESET, SSB or any other reference signal within the channel bandwidth of the cell. NOTE 3: No OCNG is transmitted from the probe transmitting non-serving beam. |  |  |

Table A.2.1-3: OP.3: Generic OCNG pattern for unused REs in the same BW as CORESET

| OCNG Parameters | Control Region | Data Region |
| --- | --- | --- |
| Resource allocation | Unused REs (Note 1) | Unused REs (Note 2) |
| Channel | PDCCH | PDSCH |
| Contents | Virtual UE IDs | Uncorrelated pseudo random QPSK modulated data |
| Antenna transmission scheme | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Subcarrier spacing | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Aggregation level | Same as used in PDCCH RMC | N/A |
| Code rate | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Transmit Power | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| CP length | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| NOTE 1: REs not used in the active CORESETs where PDCCH is scheduled for the UE under test. REs for OCNG shall not be allocated outside the allocated bandwidth of the CORESET of the serving cell. NOTE 2: REs not allocated to any physical channels, CORESET, SSB or any other reference signal within the allocated bandwidth of the CORESET of the serving cell. REs for OCNG shall not be allocated outside the allocated bandwidth of the CORESET of the serving cell. |  |  |

Table A.2.1-4: OP.4: Generic OCNG pattern for all unused REs outside SSB slot(s)

| OCNG Parameters | Control Region | Data Region |
| --- | --- | --- |
| Resource allocation | Unused REs (Note 1) | Unused REs (Note 2) |
| Channel | PDCCH | PDSCH |
| Contents | Virtual UE IDs | Uncorrelated pseudo random QPSK modulated data |
| Antenna transmission scheme | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Subcarrier spacing | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Aggregation level | Same as used in PDCCH RMC | N/A |
| Code rate | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Transmit Power | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| CP length | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| NOTE 1: REs not used in the active CORESETs where PDCCH is scheduled for the UE under test. REs for OCNG shall not be allocated in the slot(s) containing SSB of the respective cell. NOTE 2: REs not allocated to any physical channels, CORESET, SSB or any other reference signal within the channel bandwidth of the cell. REs for OCNG shall not be allocated in the slot(s) containing SSB of the respective cell. |  |  |

Table A.2.1-5: OP.5: Generic OCNG pattern for unused REs in the same BW as CORESET for 2AoA setup

| OCNG Parameters | Control Region | Data Region |
| --- | --- | --- |
| Probe | Transmitting the serving beam |  |
| Resource allocation | Unused REs (Note 1) in the symbols where SSB/CSI-RS are not transmitted from both the serving beam probe and non-serving beam probe. | Unused REs (Note 2) in the symbols where SSB/CSI-RS are not transmitted from both the serving beam probe and non-serving beam probe. |
| Channel | PDCCH | PDSCH |
| Contents | Virtual UE IDs | Uncorrelated pseudo random QPSK modulated data |
| Antenna transmission scheme | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Subcarrier spacing | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Aggregation level | Same as used in PDCCH RMC | N/A |
| Code rate | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Transmit Power | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| CP length | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| NOTE 1: REs not used in the active CORESETs where PDCCH is scheduled for the UE under test. REs for OCNG shall not be allocated outside the allocated bandwidth of the CORESET of the serving cell. NOTE 2: REs not allocated to any physical channels, CORESET, SSB or any other reference signal within the allocated bandwidth of the CORESET of the serving cell. REs for OCNG shall not be allocated outside the allocated bandwidth of the CORESET of the serving cell. NOTE 3: No OCNG is transmitted from the probe transmitting non-serving beam. |  |  |

# A.3	Reference SSB configuration

## A.3.1	SSB configuration for FR1

Table A.3.1-1: SSB allocation for FR1

| SSB Parameters | Unit | Value |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SSB Pattern |  | SSB.1 FR1 | SSB.2 FR1 | SSB.3 FR1 |  | SSB.4 FR1 |  | SSB.5 FR1 | SSB.6 FR1 |
| Channel bandwidth | MHz | 10 | 40 | 10 |  | 40 |  | 10 | 40 |
| SSB SCS | kHz | 15 | 30 | 15 |  | 30 |  | 15 | 30 |
| SSB periodicity (TSSB) | ms | 20 |  |  |  |  |  |  |  |
| Number of SSBs per SS-burst |  | 1 | 1 | 2 |  | 2 |  | 1 | 1 |
| SS/PBCH block index |  | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 0 |
| Indices of symbols containing SSB |  | 2-5 | 4-7 or 2-5 Note 2 | 2-5 | 8-11 | 4-7 or 2-5 Note 2 | 8-11 | 2-5 | 4-7 or 2-5 Note 2 |
| Indices of slots containing SSB |  | 0 |  |  |  |  |  |  |  |
| Indices of SFN containing SSB |  | SFN mod (max(TSSB,10ms)/10ms) = 0 |  |  |  |  |  | SFN mod (max(TSSB,10ms)/10ms) = 1 |  |
| RB numbers containing SSB within channel BW |  | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |  |  |  |  |  |  |  |
| NOTE 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [28]. NOTE 2: Symbols 4-7 are chosen if the SSB pattern Case B should be used for the current band as indicated by Table 5.4.3.3-1 of TS 38.104 [28]; Otherwise, symbols 2-5 are chosen. |  |  |  |  |  |  |  |  |  |

## A.3.2	SSB configuration for FR2

Table A.3.2-1: SSB allocation for FR2

| SSB Parameters | Unit | Value |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SSB Pattern |  | SSB.1 FR2 |  | SSB.2 FR2 |  |  | SSB.3 FR2 | SSB.4 FR2 | SSB.5 FR2 |  | SSB.6 FR2 |  | SSB.7 FR2 | SSB.8 FR2 |
| Channel bandwidth | MHz | 100 |  |  |  |  |  |  |  |  |  |  |  |  |
| SSB SCS | kHz | 120 |  | 240 |  |  | 120 | 240 | 120 |  | 240 |  | 120 | 240 |
| SSB periodicity (TSSB) | ms | 20 |  |  |  |  |  |  |  |  |  |  |  |  |
| Number of SSBs per SS-burst |  | 2 |  | 2 |  |  | 1 | 1 | 2 |  | 2 |  | 1 | 1 |
| SS/PBCH block index |  | 0 | 1 | 0 | 1 |  | 0 | 0 | 2 | 3 | 2 | 3 | 1 | 1 |
| Indices of symbols containing SSB |  | 4-7 | 8-11 | 8-11 | 12-13, 0-1 |  | 4-7 | 8-11 | 2-5 | 6-9 | 2-5 | 6-9 | 8-11 | 12-13, 0-1 |
| Indices of slots containing SSB |  | 0 |  | 0 |  | 0-1 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0-1 |
| Indices of SFN containing SSB |  | SFN mod (max(TSSB,10ms)/10ms) = 0 |  |  |  |  |  |  |  |  |  |  |  |  |
| RB numbers containing SSB within channel BW |  | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [28]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# A.4	Reference SMTC configuration

Table A.4-1: SMTC configurations

| SMTC Parameters | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SMTC Pattern |  | SMTC.1 | SMTC.2 | SMTC.3 | SMTC.4 | SMTC.5 | SMTC.6 |
| SMTC periodicity | ms | 20 | 20 | 160 | 20 | 20 | 20 |
| SMTC offset | ms | 0 | 0 | 0 | 10 | 10 | 17 |
| SMTC duration | ms | 1 | 5 | 1 | 1 | 5 | 5 |

# A.5	Reference DRX configurations

The reference DRX configurations for the NR serving cell are captured in Table A.5-1. The reference DRX configurations for the E-UTRA serving cell for NSA and inter-RAT test cases are captured in Table A.5-2.

Table A.5-1: DRX configurations for NR serving cell

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRX Configuration |  | DRX.1 | DRX.2 | DRX.3 | DRX.6 | DRX.7 | DRX.8 | DRX.11 |
| drx-onDurationTimer | ms | 1 | 1 | 6 | 1 | 6 | 6 | 6 |
| drx-InactivityTimer | ms | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| drx-RetransmissionTimerDL | slot | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| drx-RetransmissionTimerUL | slot | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| drx-LongCycleStartOffset | ms | 40 | 640 | 40 | 320 | 640 | 320 | 20 |
| shortDRX | - | disabled | disabled | disabled | disabled | disabled | disabled | disabled |
| TimeAlignmentTimer | ms | 500 | 500 | Infinity | 500 | Infinity | Infinity | Infinity |
| NOTE 1: The DRX cycle and time alignment timer parameters are specified in clause 6.3.2 in TS 38.331 [13]. |  |  |  |  |  |  |  |  |

Table A.5-2: DRX configurations for E-UTRA serving cell

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| DRX Configuration |  | DRX.4 | DRX.5 | DRX.9 | DRX.10 | DRX.12 |
| drx-onDurationTimer | ms | psf2 | psf6 | psf2 | psf6 | psf6 |
| drx-InactivityTimer | ms | psf2 | psf1920 | psf100 | psf1920 | psf2 |
| drx-RetransmissionTimerDL | slot | psf16 | psf16 | psf16 | psf16 | psf16 |
| drx-LongCycleStartOffset | ms | sf160, 0 | sf320. 0 | sf40, 0 | sf640. 0 | sf640, 0 |
| shortDRX | - | disabled | disabled | disabled | disabled | disabled |
| TimeAlignmentTimer | ms | Infinity | Infinity | 500 | 500 | Infinity |
| NOTE 1: The DRX cycle and time alignment timer parameters are specified in clause 6.3.2 in TS 36.331 [29] |  |  |  |  |  |  |  |

# A.6	EN-DC test setup

The purpose of this Annex is to specify the EN-DC configuration for the test cases in Chapters 4 and 5 of this test specification.

## A.6.1	E-UTRA serving cell parameters

This clause defines the cell power levels and other specific cell parameters of the E-UTRA serving cell for EN-DC.

### A.6.1.1	E-UTRA serving cell parameters for EN-DC tests with NR FR1

Table A.6.1.1-1 defines the E-UTRA serving cell parameters for EN-DC tests with NR FR1 cell(s), defined in Chapter 4 of this test specification and for iRAT tests with E-UTRA serving cell and NR FR1 cell(s) defined in Chapter 8 of this test specification. Unless otherwise stated within the test, all measurements in Clauses 4 and 5 are performed only on the NR carrier. The E-UTRA PCell shall configured to not interfere with NR operation and the E-UTRA PCell signal power shall not be critical to the test purpose.

Table A.6.1.1-1: E-UTRAN cell specific test parameters for EN-DC tests with NR FR1

| Parameter | Unit | E-UTRAN Cell1 |
| --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 |
| Duplex mode |  | FDD or TDD |
| TDD special subframe configurationNote1 |  | 6 |
| TDD uplink-downlink configurationNote1 |  | 1 |
| BWchannel Note 6 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |
| PDSCH parameters: DL Reference Measurement ChannelNote2, Note 6 |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2, Note 6 |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |
| OCNG PatternsNote 2 |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |
| PBCH_RA | dB | 0 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PHICH_RA | dB |  |
| PHICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| PDSCH_RA | dB |  |
| PDSCH_RB | dB |  |
| OCNG_RANote 3 | dB |  |
| OCNG_RBNote 3 | dB |  |
| NocNote 4 | dBm/15 kHz | -104 |
| Ês/Noc | dB | 17 |
| Ês/Iot | dB | 17 |
| RSRP Note 5 | dBm/15 kHz | -87 |
| SCH_RP Note 5 | dBm/15 kHz | -87 |
| Io Note 5 | dBm/Ch BW | -59.13 + 10log(NRB,c /50) |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 1x2 |
| NOTE 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. NOTE 2: DL RMCs and OCNG patterns are specified in clauses A.1, A.2 and D.1 of TS 36.521-3 [26]. NOTE 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 5: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 6: For E-UTRA anchor configuration, pick 5 MHz as default channel bandwidth setting in the tests as it is supported by all E-UTRA bands. If none of the UE supported EN-DC band combos support 5MHz E-UTRA carrier, pick 20 MHz channel BW or 10 MHz channel BW, in that order, |  |  |

Table A.6.1.1-2: CQI-ReportConfig-DEFAULT: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-2 CQI-ReportConfig-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic | NOT PRESENT |  |  |
| cqi-ReportPeriodic | NOT PRESENT |  |  |
| } |  |  |  |

Table A.6.1.1-3:PhysicalConfigDedicated-DEFAULT: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.8.2, Table 4.8.2.1.6-1 PhysicalConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |  |
| soundingRS-UL-ConfigDedicated | Not present |  | RBC |  |
| } |  |  |  |  |

Table A.6.1.1-4:MAC-MainConfig-RBC: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.8.2.1.5, Table 4.8.2.1.5-1 MAC-MainConfig-RBC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| timeAlignmentTimerDedicated | Infinity |  |  |

### A.6.1.2	E-UTRA serving cell parameters for EN-DC tests with NR FR2

Table A.6.1.2-1 defines the E-UTRA serving cell parameters for EN-DC tests with NR FR2 cell(s), defined in Chapter 5 of this test specification and for iRAT tests with E-UTRA serving cell and NR FR1 cell(s) defined in Chapter 8 of this test specification. Unless otherwise stated within the test, all measurements in Clauses 5 and 7 are performed only on the NR carrier. The E-UTRA PCell shall configured to not interfere with NR operation and the E-UTRA PCell signal power shall not be critical to the test purpose.

Table A.6.1.2-1: E-UTRAN cell specific test parameters for EN-DC tests with NR FR2

| Parameter | Unit | E-UTRAN Cell1 |
| --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 |
| Duplex mode |  | FDD or TDD |
| TDD special subframe configurationNote1 |  | 6 |
| TDD uplink-downlink configurationNote1 |  | 1 |
| BWchannel Note 5 | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |
| PDSCH parameters: DL Reference Measurement ChannelNote2, Note 5 |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2, Note 5 |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |
| OCNG PatternsNote2, Note 5 |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |
| PBCH_RA | dB | 0 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PHICH_RA | dB |  |
| PHICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| PDSCH_RA | dB |  |
| PDSCH_RB | dB |  |
| OCNG_RANote3 | dB |  |
| OCNG_RBNote3 | dB |  |
| NOTE 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. NOTE 2: DL RMCs and OCNG patterns are specified in clauses A 1, A.2 and D.1 of TS 36.521-3 [26]. NOTE 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 4: The E-UTRA signal is required only to ensure the E-UTRA link to the DUT in the EN-DC operation. The Test System shall provide a stable and noise-free E-UTRA signal without need of precise propagation modelling, path loss and polarization control. Further details of the E-UTRA signal configuration are not defined as part of the cell specific test parameters, since the E-UTRA link is not under performance verification and is not expected to influence the NR FR2 requirement. NOTE 5: The For E-UTRA anchor configuration, pick 5 MHz as default channel bandwidth setting in the tests as it is supported by all E-UTRA bands. If none of the UE supported EN-DC band combos support 5MHz E-UTRA carrier, pick 20 MHz channel BW or 10 MHz channel BW, in that order, |  |  |

Table A.6.1.2-2: CQI-ReportConfig-DEFAULT: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-2 CQI-ReportConfig-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic | NOT PRESENT |  |  |
| cqi-ReportPeriodic | NOT PRESENT |  |  |
| } |  |  |  |

Table A.6.1.2-3:PhysicalConfigDedicated-DEFAULT: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.8.2, Table 4.8.2.1.6-1 PhysicalConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |  |
| soundingRS-UL-ConfigDedicated | Not present |  | RBC |  |
| } |  |  |  |  |

Table A.6.1.2-4:MAC-MainConfig-RBC: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.8.2.1.5, Table 4.8.2.1.5-1 MAC-MainConfig-RBC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| timeAlignmentTimerDedicated | Infinity |  |  |

# A.6A	NR FR1-FR2 test setup

Some test cases in clause 7 have NR cells in both FR1 and FR2. Unless otherwise stated within the test, the NR FR1 Cell signal is required only to provide a link to the UE under test. The Test Equipment shall provide a stable and noise-free NR FR1 signal without need of precise propagation modelling, path loss and polarization control. Further details of the NR FR1 signal configuration are not defined as part of the cell specific test parameters, since the NR FR1 link is not under performance verification and is not expected to influence the test purpose.

# A.7	Reference PRACH configurations

This clause provides the typical PRACH configurations used for all RRM test cases defined in this test specification. Parameters not listed in this clause can be derived from the configuration of each test.

## A.7.1	PRACH configurations for FR1

Table A.7.1-1 defines the PRACH configurations for FR1. Each of the PRACH configurations defined in Table A.7.1-1 have different applicabilities:

-	PRACH.1 FR1 for SSB-based contention based random access in FR1.

-	PRACH.2 FR1 for SSB-based non-contention based random access in FR1.

-	PRACH.3 FR1 for CSI-RS based non-contention based random access in FR1.

-	PRACH.4 FR1 for CSI-RS based non-contention based random access in FR1 to convey BFR.

Table A.7.1-1 Parameters for PRACH Configurations for FR1

| Field | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- |
| PRACH Configuration | PRACH.1 FR1 | PRACH.2 FR1 | PRACH.3 FR1 | PRACH.4 FR1 |  |
| prach-ConfigurationIndex | 102 | 102 | 102 | 102 | 10ms PRACH periodicity and other detailed configuration defined in table 6.3.3.2-2 in TS 38.211 [7]. |
| msg1-SubcarrierSpacing | Same as UL carrier SCS | Same as UL carrier SCS | Same as UL carrier SCS | Same as UL carrier SCS |  |
| totalNumberOfRA-Preambles | 48 | 48 | 48 | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | 48 | 48 | 48 | No group B. |
| prach-RootSequenceIndex | 0 | 0 | 0 | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB | oneFourth, n48 | - | - | - | OneFourth: 1 SSB associated with 4 RACH occasions n48: 48 contention based preambles per SSB |
| ssb-perRACH-Occasion | - | oneFourth | oneFourth | oneFourth | OneFourth: 1 SSB associated with 4 RACH occasions |
| msg1-FDM | One | One | One | One | One PRACH transmission occasions FDMed in one time instance. |
| rsrp-ThresholdSSB | RSRP_51 | RSRP_51 | N/A | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [13]. |
| rsrp-ThresholdCSI-RS | N/A | N/A | RSRP_51 | N/A |  |
| ra-ContentionResolutionTimer | sf48 | - | - | - | 48 sub-frames |
| powerRampingStep | dB2 | dB2 | dB2 | dB2 |  |
| preambleReceivedTargetPower | dBm-120 | dBm-120 | dBm-120 | dBm-120 |  |
| preambleTransMax | n6 | n6 | n6 | n200 | Max number of RA preamble transmission performed before declaring a failure is 6 |
| ra-ResponseWindow | sl10 | sl10 | sl10 | sl1 | 10 slots |
| zeroCorrelationZoneConfig | 11 | 11 | 11 | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 2 | 2 | 2 | 20ms, as defined in table 7.2-1 in TS 38.321 [12]. |
| ssb-ResourceList | - | present | N/A | N/A | Associated with SSB index 0 |
| ra-PreambleIndex | - | 50 | N/A | N/A | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE doesn’t use this field if is transmitting CFRA to convey BFR. |
| BFR-SSB-Resource | - | present | N/A | N/A | Associated with SSB index 0 |
| ra-PreambleIndex | - | 50 | N/A | N/A | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE uses this field only if is transmitting CFRA to convey BFR |
| csirs-ResourceList | N/A | N/A | present | presnt | Associated with CSI-RS configured |
| ra-PreambleIndex | N/A | N/A | 50 | 50 | Associated with CSI-RS configured |
| ra-OccasionList | - | - | 1 | 1 | RA occasions allowed corresponding to CSI-RS |
| ra-ssb-OccasionMaskIndex | - | 1 | N/A | N/A | PRACH occasion index 1 is allowed |
| NOTE: For further information see Clause 6.3.2 in TS 38.331 [13]. |  |  |  |  |  |

## A.7.2	PRACH configurations for FR2

Table A.7.2-1 defines the PRACH configurations for FR2. Each of the PRACH configurations defined in Table A.7.2-1 have different applicabilities:

-	PRACH.1 FR2 for SSB-based contention based random access in FR2.

-	PRACH.2 FR2 for SSB-based non-contention based random access in FR2.

-	PRACH.3 FR2 for CSI-RS based non-contention based random access in FR2.

-	PRACH.4 FR2 for CSI-RS based non-contention based random access in FR2 to convey BFR.

Table A.7.2-1 Parameters for PRACH Configurations for FR2

| Field | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- |
| PRACH Configuration | PRACH.1 FR2 | PRACH.2 FR2 | PRACH.3 FR2 | PRACH.4 FR2 |  |
| prach-ConfigurationIndex | 190 | 190 | 190 | 190 | Preamble format C2, 10ms PRACH periodicity and other detailed configuration defined in table 6.3.3.2-4 in TS 38.211 [7]. |
| msg1-SubcarrierSpacing | Same as UL carrier SCS | Same as UL carrier SCS | Same as UL carrier SCS | Same as UL carrier SCS |  |
| totalNumberOfRA-Preambles | 48 | 48 | 48 | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | 48 | 48 | 48 | No group B. |
| prach-RootSequenceIndex | 0 | 0 | 0 | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB | oneFourth, n48 | N/A | N/A | N/A | OneFourth: 1 SSB associated with 4 RACH occasions n48: 48 contention based preambles per SSB |
| ssb-perRACH-Occasion | N/A | oneFourth | oneFourth | oneFourth | OneFourth: 1 SSB associated with 4 RACH occasions |
| msg1-FDM | One | One | One | One | One PRACH transmission occasions FDMed in one time instance. |
| rsrp-ThresholdSSB | RSRP_51 | RSRP_51 | N/A | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [13]. |
| rsrp-ThresholdCSI-RS | N/A | N/A | RSRP_51 | N/A |  |
| ra-ContentionResolutionTimer | sf48 | N/A | N/A | N/A | 48 sub-frames |
| powerRampingStep | dB2 | dB2 | dB2 | dB2 |  |
| preambleReceivedTargetPower | dBm-120 | dBm-120 | dBm-120 | dBm-120 |  |
| preambleTransMax | n6 | n6 | n6 | n200 | Max number of RA preamble transmission performed before declaring a failure |
| ra-ResponseWindow | sl10 | sl10 | sl10 | sl40 |  |
| zeroCorrelationZoneConfig | 11 | 11 | 11 | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 2 | 2 | 2 | 20ms, as defined in table 7.2-1 in TS 38.321 [12]. |
| ssb-ResourceList | - | present | N/A | N/A | Associated with SSB index 0 |
| ra-PreambleIndex | - | 50 | N/A | N/A | Associated with SSB index 0 |
| csirs-ResourceList | N/A | present | present | present | Associated with CSI-RS configured |
| ra-PreambleIndex | N/A | 50 | 50 | 50 | Associated with CSI-RS configured |
| ra-OccasionList | - | - | 1 | 1 | RA occasions allowed corresponding to CSI-RS |
| ra-ssb-OccasionMaskIndex | - | 1 | N/A | N/A | PRACH occasion index 1 is allowed |
| NOTE: For further information see Clause 6.3.2 in TS 38.331 [13]. |  |  |  |  |  |

# A.7A	Reference MsgA configurations

This clause provides the typical PRACH and PUSCH configurations for MsgA used for all RRM test cases defined in this test specification. Parameters not listed in this clause can be derived from the configuration of each test.

## A.7A.1	MsgA configurations for FR1

Table A.7A.1-1 defines the MsgA configurations for FR1. Each of the MsgA configurations defined in Table A.7A.1-1 have different applicabilities:

-	MsgA.1 FR1 for SSB-based 2-step contention based random access in FR1.

-	MsgA.2 FR1 for SSB-based 2-step non-contention based random access in FR1.

Table A.7A.1-1 Parameters for MsgA Configurations for FR1

| Field | Value |  | Comment |
| --- | --- | --- | --- |
| MsgA Configuration | MsgA.1 FR1 | MsgA.2 FR1 |  |
| msgA-prach-ConfigurationIndex | 102 | 102 | 10ms PRACH periodicity and other detailed configuration defined in table 6.3.3.2-2 in TS 38.211 [7]. |
| msgA-SubcarrierSpacing | Same as UL carrier SCS | Same as UL carrier SCS |  |
| msgA-totalNumberOfRA-Preambles | 48 | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | 48 | No group B. |
| msgA-PRACH-RootSequenceIndex | 0 | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| msgA-SSB-perRACH-OccasionAndCB-PreamblesPerSSB | oneFourth, n48 | oneFourth | OneFourth: 1 SSB associated with 4 RACH occasions |
| msgA-RO-FDM | One | One | One PRACH transmission occasions FDMed in one time instance. |
| ra-ContentionResolutionTimer | sf48 | - | 48 sub-frames |
| msgA-PreamblePowerRampingStep | dB2 | dB2 |  |
| msgA-PreambleReceivedTargetPower | dBm-120 | dBm-120 |  |
| preambleTransMax | n6 | n6 | Max number of RA preamble transmission performed before declaring a failure is 6 |
| msgB-ResponseWindow | sl10 | sl10 | 10 slots |
| msgA-ZeroCorrelationZoneConfig | 11 | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 2 | 20ms, as defined in table 7.2-1 in TS 38.321 [7]. |
| ssb-ResourceList | - | ra-PreambleIndex = 50 | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE doesn’t use this field if is transmitting CFRA to convey BFR. |
| BFR-SSB-Resource | - | ra-PreambleIndex = 50 | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE uses this field only if is transmitting CFRA to convey BFR |
| ra-ssb-OccasionMaskIndex | - | 1 | PRACH occasion index 1 is allowed |
| msgA-MCS | 1 | 1 | MCS index for MsgA PUSCH |
| nrofSlotsMsgA-PUSCH | 1 | 1 | Number of slots containing one or multiple PUSCH occasions |
| nrofMsgA-PO-PerSlot | 1 | 1 | Number of time domain PUSCH occasions in each slot |
| msgA-PUSCH-TimeDomainOffset | 1 | 1 | A single time offset with respect to the start of each PRACH slot, counted as the number of slots |
| PUSCH start symbol | 0 | 0 |  |
| PUSCH allocation length | 14 | 14 |  |
| mappingTypeMsgA-PUSCH | typeA | typeA |  |
| nrofPRBs-PerMsgA-PO | 2 | 2 | Number of RBs per PUSCH occasion |
| nrofMsgA-PO-FDM | One | One | The number of MsgA PUSCH occasions FDMed in one time instance |
| msgA-DMRS-AdditionalPosition | pos1 | pos1 | Position for additional DM-RS |
| msgA-PUSCH-NrofPorts | 1 | 1 | Configure 1 port per CDM group |
| msgA-DeltaPreamble | 3 | 3 | Power offset of msgA PUSCH relative to the preamble received target power |
| msgA-Alpha | alpha1 | alpha1 | Alpha value for MsgA PUSCH. Set 1 |
| deltaMCS | Disabled | Disabled | Whether to apply delta MCS |
| NOTE: For further information see clause 6.3.2 in TS 38.331 [13]. |  |  |  |

## A.7A.2	MsgA configurations for FR2

Table A.7A.2-1 defines the MsgA configurations for FR2. Each of the MsgA configurations defined in Table A.7A.2-1 have different applicabilities:

-	MsgA.1 FR2 for SSB-based contention based random access in FR2.

-	MsgA.2 FR2 for SSB-based non-contention based random access in FR2.

Table A.7A.2-1 Parameters for MsgA Configurations for FR2

| Field | Value |  | Comment |
| --- | --- | --- | --- |
| MsgA Configuration | MsgA.1 FR2 | MsgA.2 FR2 |  |
| msgA-prach-ConfigurationIndex | 190 | 190 | Preamble Format C2, 10ms PRACH periodicity and other detailed configuration defined in table 6.3.3.2-2 in TS 38.211 [7]. |
| msgA-SubcarrierSpacing | Same as UL carrier SCS | Same as UL carrier SCS |  |
| msgA-totalNumberOfRA-Preambles | 48 | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | 48 | No group B. |
| msgA-PRACH-RootSequenceIndex | 0 | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| msgA-SSB-perRACH-OccasionAndCB-PreamblesPerSSB | oneFourth, n48 | oneFourth | OneFourth: 1 SSB associated with 4 RACH occasions |
| msgA-RO-FDM | One | One | One PRACH transmission occasions FDMed in one time instance. |
| ra-ContentionResolutionTimer | sf48 | - | 48 sub-frames |
| msgA-PreamblePowerRampingStep | dB2 | dB2 |  |
| msgA-PreambleReceivedTargetPower | dBm-120 | dBm-120 |  |
| preambleTransMax | n6 | n6 | Max number of RA preamble transmission performed before declaring a failure is 6 |
| msgB-ResponseWindow | sl10 | sl10 | 10 slots |
| msgA-ZeroCorrelationZoneConfig | 11 | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 2 | 20ms, as defined in table 7.2-1 in TS 38.321 [7]. |
| ssb-ResourceList | - | ra-PreambleIndex = 50 | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE doesn’t use this field if is transmitting CFRA to convey BFR. |
| BFR-SSB-Resource | - | ra-PreambleIndex = 50 | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE uses this field only if is transmitting CFRA to convey BFR |
| ra-ssb-OccasionMaskIndex | - | 1 | PRACH occasion index 1 is allowed |
| msgA-MCS | 1 | 1 | MCS index for MsgA PUSCH |
| nrofSlotsMsgA-PUSCH | 1 | 1 | Number of slots containing one or multiple PUSCH occasions |
| nrofMsgA-PO-PerSlot | 1 | 1 | Number of time domain PUSCH occasions in each slot |
| msgA-PUSCH-TimeDomainOffset | 1 | 1 | A single time offset with respect to the start of each PRACH slot, counted as the number of slots |
| PUSCH start symbol | 0 | 0 |  |
| PUSCH allocation length | 10 | 10 |  |
| mappingTypeMsgA-PUSCH | typeA | typeA |  |
| nrofPRBs-PerMsgA-PO | 2 | 2 | Number of RBs per PUSCH occasion |
| nrofMsgA-PO-FDM | One | One | The number of MsgA PUSCH occasions FDMed in one time instance |
| msgA-DMRS-AdditionalPosition | pos1 | pos1 | Position for additional DM-RS |
| msgA-PUSCH-NrofPorts | 1 | 1 | Configure 1 port per CDM group |
| msgA-DeltaPreamble | 3 | 3 | Power offset of msgA PUSCH relative to the preamble received target power |
| msgA-Alpha | alpha1 | alpha1 | Alpha value for MsgA PUSCH. Set 1 |
| deltaMCS | Disabled | Disabled | Whether to apply delta MCS |
| NOTE: For further information see clause 6.3.2 in TS 38.331 [13]. |  |  |  |

# A.8	Reference BWP configurations

This clause provides the typical BWP configurations used for RRM test cases defined in this test specification. For downlink BWP, both initial BWP and dedicated BWP configurations are specified in clause A.8.1 and for uplink BWP, dedicated BWP configurations are specified in clause A.8.2. Parameters not listed in this clause can be derived from the configuration of each test.

## A.8.1	Downlink BWP configurations

Table A.8.1-1 defines the different downlink initial BWP configurations. Table A.8.1-2 defines the different downlink dedicated BWP configurations.

Table A.8.1-1: Downlink BWP patterns for initial BWP configuration

| BWP Parameters | Unit | Values |  |  |
| --- | --- | --- | --- | --- |
| DL BWP |  | DLBWP.0.1 | DLBWP.0.2 |  |
| Starting PRB index |  | 0 | RBc Note 1 |  |
| Bandwidth |  | Same as RF channel defined in each test | same as RMSI CORSET(CORESET #0) defined in each test |  |
| NOTE 1: RBc is the lowest PRB index to guarantee the BWP including CORESET #0 which is defined in Clause A.1.2. |  |  |  |  |

Table A.8.1-2: Downlink BWP patterns for dedicated BWP configuration

| BWP Parameters | Unit | Values |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL BWP |  | DLBWP.1.1 | DLBWP.1.2 |  |  |  | DLBWP.1.3 |  |  |  |
| Starting PRB index |  | 0 | RBb Note 1 |  |  |  | RBa Note 2 |  |  |  |
| SSB SCS | kHz |  | 15 | 30 | 120 | 240 | 15 | 30 | 120 | 240 |
| Bandwidth | RB | Same as RF channel defined for the serving cell in each test | 25 | 51 | 32 | 48 | 25 | 51 | 32 | 48 |
| NOTE 1: RBb is the lowest PRB index to guarantee the BWP not fully overlapped with SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3. NOTE 2: RBa is the lowest PRB index to guarantee the BWP including SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3. |  |  |  |  |  |  |  |  |  |  |

## A.8.2	Uplink BWP configurations

Table A.8.2-1 defines the uplink initial BWP configurations. Table A.8.2-2 defines the uplink dedicated BWP configurations.

Table A.8.2-1: Uplink BWP patterns for initial BWP configurations

| BWP Parameters | Values |  |
| --- | --- | --- |
| UL BWP | ULBWP.0.1 | ULBWP.0.2 |
| Staring PRB index | 0 | RBc Note 1 |
| Bandwidth | Same as RF channel defined in each test | same as RMSI CORESET(CORSET #0) defined in each test |
| NOTE 1: RBc is the lowest PRB index to guarantee the BWP including CORESET #0 which is defined in Clause A.1.2. |  |  |

Table A.8.2-2: Uplink BWP patterns for dedicated BWP configurations

| BWP Parameters | Unit | Values |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL BWP |  | ULBWP.1.1 | ULBWP.1.2 |  |  |  | ULBWP.1.3 |  |  |  |
| Staring PRB index |  | 0 | RBb Note 1 |  |  |  | RBa Note 1 |  |  |  |
| SSB SCS | kHz |  | 15 | 30 | 120 | 240 | 15 | 30 | 120 | 240 |
| Bandwidth | RB | Same as RF channel defined for the serving cell in each test | 25 | 51 | 32 | 48 | 25 | 51 | 32 | 48 |
| NOTE 1: RBb is the lowest PRB index to guarantee the BWP not fully overlapped with SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3. NOTE 2: RBa is the lowest PRB index to guarantee the BWP including SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Clause A.3. |  |  |  |  |  |  |  |  |  |  |

# A.9	Angle of Arrival (AoA) for FR2 RRM test cases

This clause specifies the AoA setups for FR2 RRM test cases in clause 5 and 7. The applicable AoA setup is defined in each test case in clause 5 and 7.

## A.9.1	Setup 1: Single AoA in Rx beam peak direction

There is only one active probe in the test. The DL signals, and noise if applicable, transmitted from the probe, are aligned to the UE Rx beam peak direction (as defined in TS 38.101-2 [3]).

## A.9.2	Setup 2: Single AoA in non Rx beam peak direction

### A.9.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction

There is only one active probe in the test. The DL signals, and noise if applicable, transmitted from the probe, align to a direction (AoA) which is from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [3] for each UE power class. The direction (AoA) of the signals shall not be changed between test iterations.

### A.9.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction

There is only one active probe in the test. The DL signals, and noise if applicable, transmitted from the probe, align to a direction (AoA) which is from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [3] for each UE power class. For UE power class 3, the direction (AoA) of the signals shall be changed for each test iteration (for UE power classes other than 3, this is FFS). Unless otherwise specified by the test case, the TE shall alternate between a minimum of 33 different AoAs. The minimum separation between AoAs used in consecutive iterations shall be 30° in great-circle distance.

NOTE:	if it is not possible to find at least 33 different test points meeting the required criteria (FFS), the test shall alternate between all the available test points.

## A.9.3	Setup 3: 2 AoAs

There are 2 active probes in the test. The DL signals, and noise if applicable, transmitted from the two active probes, align to directions (AoAs) which are from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [3] for each UE power class. The relative angular offset between the directions (AoAs) of the 2 active probes, shall be changed for each test iteration. Unless otherwise specified by the test case, the TE shall alternate between a minimum of 33 different test points. A test point comprehends the relative angular offset between the active probes, and the AoA of each of the signals from the UE point of view. The relative angular offset between the active probes must change for consecutive test points.

The applicable set of relative angular offsets between the 2 active probes is given in Table A.9.3-1 for each UE power class.

Table A.9.3-1: Set of relative angular offsets between active probes for each power class

| UE Power class | Relative angular offset between active probes |
| --- | --- |
| 1 | FFS |
| 2 | FFS |
| 3 | 30°, 60°, 90°, 120° and 150° |
| 4 | FFS |

NOTE:	if it is not possible to find at least 33 different test points meeting the required criteria (that is, the two AoAs are from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [3] and the angular offset between them corresponds to one of the offsets in Table A.9.3-1 for the corresponding UE power class), the test shall alternate between all the available test points.

## A.9.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak

### A.9.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction

There are 2 active probes in the test. The DL signals, and noise if applicable, are transmitted from the two active probes. One probe is aligned to the UE Rx beam peak direction as defined in TS 38.101-2 [3]. The second is aligned to a direction (AoA) which is from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [3] for each UE power class. The direction (AoA) of the non Rx beam peak signal shall not be changed between test iterations.

### A.9.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction

There are 2 active probes in the test. The DL signals, and noise if applicable, are transmitted from the two active probes. One probe is aligned to the UE Rx beam peak direction as defined in TS 38.101-2 [3]. The second is aligned to a direction (AoA) which is from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [3] for each UE power class.

For UE power class 3, the relative angular offset between the directions (AoAs) of the 2 active probes shall be changed for each test iteration, within the probe alignment described above. The applicable set of relative angular offsets between the 2 active probes is given in Table A.9.3-1 for each UE power class. Unless otherwise specified by the test case, the TE shall alternate between a minimum of 5 different test points. A test point comprehends the relative angular offset between the active probes, and the AoA of each of the signals from the UE point of view, one of them being the Rx beam peak direction. The relative angular offset between the active probes must change for consecutive test points.

NOTE:	if it is not possible to find at least 5 different test points meeting the required criteria (that is, the two AoAs are from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [3] and the angular offset between them corresponds to one of the offsets in Table A.9.3-1 for the corresponding UE power class), the test shall alternate between all the available test points.

# A.10	TCI State Configuration

## A.10.1	Introduction

This clause provides the configurations for TCI states towards either SSB or CSI-RS. The TCI states defined in this clause are configured in each test when applicable to indicate that certain DL signals are quasi-collocated with the referenceSignal configured in the TCI states.

## A.10.2	TCI states

Table A.10.2-1: TCI States

| Parameter | TCI.State.0 | TCI.State.1 | TCI.State.2 | TCI.State.3 |
| --- | --- | --- | --- | --- |
| tci-StateId | Id0 | Id1 | Id2 | Id3 |
| qcl-Type1 | typeC | typeC | typeA | typeA |
| qcl-Type2Note1 | typeD | typeD | typeD | typeD |
| referenceSignal | SSB0 | SSB1 | Resource #4 in TRS resource set 1 Note3 | Resource #4 in TRS resource set 2 Note3 |
| NOTE 1: qcl-Type2 of typeD only where applicable. For RRM test cases, this will be only in FR2 NOTE 2: referenceSignal configurations towards which the TCI states are configured are defined in a test-specific manner. NOTE 3: Reference TRS resource sets are defined in A.3.17, and the applicable TRS resource set(s) are specified in each test case. When a single TRS resource set is configured in a test case, it is considered as resource set 1. |  |  |  |  |

Annex B (normative):
Conditions for RRM requirements applicability for operating bands

# B.1	Conditions for NR RRC_IDLE state mobility

## B.1.1	Introduction

In Annex B.1, the following conditions are specified:

-	UE conditions which shall apply for UE intra-frequency idle state mobility test cases in clauses 6.1 and 7.1,

-	UE conditions which shall apply for UE inter-frequency idle state mobility test cases in clauses 6.1 and 7.1.

## B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection

Editor’s notes for Table B.1.2-2 (RAN4 dependant):

- The value of Y for Power classes 1 and 4 is FFS, where Y1 and Y4 are the rough/fine beam gain differences in Rx beam peak direction for Power classes 1 and 4 respectively

- The value of Z for Power classes 1 and 4 is FFS, where Z1 and Z4 are the rough/fine beam gain differences in spherical coverage directions for Power classes 1 and 4 respectively

This clause defines the following conditions for NR intra-frequency measurements performed based on SSBs for cell re-selection: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.1.2-1 for FR1 NR cells.

The conditions are defined in Table B.1.2-2 for FR2 NR cells.

Table B.1.2-1: Conditions for intra-frequency cell re-selection in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A | -124 | -121 | -4 |
|  | NR_FDD_FR1_B | -123.5 | -120.5 |  |
|  | NR_TDD_FR1_C | -123 | -120 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 |  |
|  | NR_FDD_FR1_G | -121 | -118 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 |  |
| NOTE 1: NR operating band groups are defined in Clause 3A.4. |  |  |  |  |

Table B.1.2-2: Conditions for intra-frequency cell re-selection in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE Power class |  |  |  | UE Power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 |  |  |
|  |  | n259 |  |  | -105.5 |  |  |  |
|  |  | n260 | -122.3+Y1 |  | -106.5 | -122.8+Y4 |  |  |
|  |  | n261 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 |  |  |
|  |  | n259 |  |  | -92.7 |  |  |  |
|  |  | n260 | -114.3+Z1 |  | -93.9 | -110.8+Z4 |  |  |
|  |  | n261 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

## B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection

This clause defines the following conditions for NR inter-frequency measurements performed based on SSBs for cell re-selection: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions defined in Table B.1.2-1 for FR1 NR intra-frequency cell re-selection shall also apply for FR1 NR inter-frequency cells in this clause.

The conditions defined in Table B.1.2-2 for FR2 NR intra-frequency cell re-selection shall also apply for FR2 NR inter-frequency cells in this clause.

# B.2	Conditions for NR RRC_CONNECTED state

## B.2.1	Introduction

In Annex B.2, the following conditions are specified:

-	UE conditions which shall apply for UE intra-frequency measurement procedures and measurement performance tests in clauses 4.6, 4.7, 5.6, 5.7, 6.6, 6.7, 7.6 and 7.7,

UE conditions which shall apply for UE inter-frequency measurements procedures and requirements in Clause 9,

-	UE conditions which shall apply for UE intra-frequency measurements performance requirements in Clause 10,

-	UE conditions which shall apply for UE inter-frequency measurements performance requirements in Clause 10.

## B.2.2	Conditions for NR intra-frequency measurements

Editor’s notes for Table B.2.2-2 (RAN4 dependant):

- The value of Y for power classes 1 and 4 is FFS, where Y1 and Y4 are the rough/fine beam gain differences in Rx beam peak direction for power classes 1 and 4 respectively

- The value of Z for power classes 1 and 4 is FFS, where Z1 and Z4 are the rough/fine beam gain differences in spherical coverage directions for power classes 1 and 4 respectively

This clause defines the following conditions for NR intra-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.2-1 for FR1 NR cells.

The conditions are defined in Table B.2.2-2 for FR2 NR cells.

Table B.2.2-1: Conditions for intra-frequency measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A | -127 | -124 | -6 |
|  | NR_FDD_FR1_B | -126.5 | -123.5 |  |
|  | NR_TDD_FR1_C | -126 | -123 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -122.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -122 |  |
|  | NR_FDD_FR1_G | -124 | -121 |  |
|  | NR_FDD_FR1_H | -123.5 | -120.5 |  |
| NOTE 1: NR operating band groups are defined in Clause 3A.4. |  |  |  |  |

Table B.2.2-2: Conditions for intra-frequency measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE power class |  |  |  | UE power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-6 |
|  |  | n258 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 |  |  |
|  |  | n259 |  |  | -108.5 |  |  |  |
|  |  | n260 | -125.3+Y1 |  | -109.5 | -125.8+Y4 |  |  |
|  |  | n261 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-6 |
|  |  | n258 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 |  |  |
|  |  | n259 |  |  | -95.7 |  |  |  |
|  |  | n260 | -117.3+Z1 |  | -96.9 | -113.8+Z4 |  |  |
|  |  | n261 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and spherical coverage values are increased by ∆MBS,n the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

## B.2.3	Conditions for NR inter-frequency measurements

Editor’s notes for Table B.2.3-2 (RAN4 dependant):

- The value of Y for power classes 1 and 4 is FFS, where Y1 and Y4 are the rough/fine beam gain differences in Rx beam peak direction for power classes 1 and 4 respectively

- The value of Z for power classes 1 and 4 is FFS, where Z1, and Z4 are the rough/fine beam gain differences in spherical coverage directions for power classes 1 and 4 respectively

This clause defines the following conditions for NR inter-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.3-1 for FR1 NR cells.

The conditions are defined in Table B.2.3-2 for FR2 NR cells.

Table B.2.3-1: Conditions for inter-frequency measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A | -125 | -122 | -4 |
|  | NR_FDD_FR1_B | -124.5 | -121.5 |  |
|  | NR_TDD_FR1_C | -124 | -121 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -124.5 | -120.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -123 | -120 |  |
|  | NR_FDD_FR1_G | -122 | -119 |  |
|  | NR_FDD_FR1_H | -121.5 | -118.5 |  |
| NOTE 1: NR operating band groups are defined in Clause 3A.4. |  |  |  |  |

Table B.2.3-2: Conditions for inter-frequency measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE power class |  |  |  | UE power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -126.3+Y1 | -111.8 | -110.1 | -125.8+Y4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -126.3+Y1 | -111.8 | -110.1 | -125.8+Y4 |  |  |
|  |  | n259 |  |  | -106.5 |  |  |  |
|  |  | n260 | -123.3+Y1 |  | -107.5 | -123.8+Y4 |  |  |
|  |  | n261 | -126.3+Y1 | -111.8 | -110.1 | -125.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -118.3+Z1 | -100.8 | -99.2 | -116.8+Z4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -118.3+Z1 | -100.8 | -99.2 | -116.8+Z4 |  |  |
|  |  | n259 |  |  | -93.7 |  |  |  |
|  |  | n260 | -115.3+Z1 |  | -94.9 | -111.8+Z4 |  |  |
|  |  | n261 | -118.3+Z1 | -100.8 | -99.2 | -116.8+Z4 |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

## B.2.4	Conditions for NR L1-RSRP reporting

### B.2.4.1	Conditions for SSB based L1-RSRP reporting

Editor’s notes for Table B.2.4.1-2 (RAN4 dependant):

- The value of Y for power classes 1 and 4 is FFS, where Y1 and Y4 are the rough/fine beam gain differences in Rx beam peak direction for power classes 1 and 4 respectively

- The value of Z for power classes 1 and 4 is FFS, where Z1 and Z4 are the rough/fine beam gain differences in spherical coverage directions for power classes 1 and 4 respectively

This clause defines the following conditions for NR L1-RSRP measurement reporting and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.4.1-1 for FR1 NR cells.

The conditions are defined in Table B.2.4.1-2 for FR2 NR cells.

Table B.2.4.1-1: Conditions for SSB based L1-RSRP measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -121 | -3 |
|  | NR_FDD_FR1_B | -123.5 | -120.5 |  |
|  | NR_TDD_FR1_C | -123 | -120 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 |  |
|  | NR_FDD_FR1_G | -121 | -118 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3A.4 |  |  |  |  |

Table B.2.4.1-2: Conditions for SSB based L1-RSRP measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE power class |  |  |  | UE power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-3 |
|  |  | n258 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 |  |  |
|  |  | n259 |  |  | -105 |  |  |  |
|  |  | n260 | -122.3+Y1 |  | -106.5 | -122.8+Y4 |  |  |
|  |  | n261 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-3 |
|  |  | n258 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 |  |  |
|  |  | n259 |  |  | -92.3 |  |  |  |
|  |  | n260 | -114.3+Z1 |  | -93.9 | -110.8+Z4 |  |  |
|  |  | n261 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

### B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting

Editor’s notes for Table B.2.4.2-2 (RAN4 dependant):

- The value of Y for power classes 1 and 4 is FFS, where Y1 and Y4 are the rough/fine beam gain differences in Rx beam peak direction for power classes 1 and 4 respectively

- The value of Z for power classes 1 and 4 is FFS, where Z1 and Z4 are the rough/fine beam gain differences in spherical coverage directions for power classes 1 and 4 respectively

This clause defines the following conditions for NR L1-RSRP measurement reporting and corresponding procedures performed based on CSI-RS: CSI-RS_RP and CSI-RS Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.4.2-1 for FR1 NR cells.

The conditions are defined in Table B.2.4.2-2 for FR2 NR cells.

Table B.2.4.2-1: Conditions for CSI-RS based L1-RSRP measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum CSI-RS_RP |  |  | CSI-RS Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm / SCSCSI-RS |  |  | dB |
|  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -121 | -118 | -3 |
|  | NR_FDD_FR1_B | -123.5 | -120.5 | -117.5 |  |
|  | NR_TDD_FR1_C | -123 | -120 | -117 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 | -116.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 | -116 |  |
|  | NR_FDD_FR1_G | -121 | -118 | -115 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 | -114.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |  |

Table B.2.4.2-2: Conditions for CSI-RS based L1-RSRP measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum CSI-RS_RP Note 2, Note 3 |  |  |  |  | CSI-RS Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSCSI-RS |  |  |  |  | dB |
|  |  |  | SCSCSI-RS = 60 kHz |  |  |  | SCSCSI-RS = 120 kHz |  |
|  |  |  | UE power class |  |  |  | UE power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 | (Value for SCSCSI-RS = 60 kHz) +3dB | ≥-3 |
|  |  | n258 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 |  |  |
|  |  | n259 |  |  | -108.5 |  |  |  |
|  |  | n260 | -125.3+Y1 |  | -109.5 | -125.8+Y4 |  |  |
|  |  | n261 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 | (Value for SCSCSI-RS = 60 kHz) +3dB | ≥-3 |
|  |  | n258 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 |  |  |
|  |  | n259 |  |  | -95.7 |  |  |  |
|  |  | n260 | -117.3+Z1 |  | -96.9 | -113.8+Z4 |  |  |
|  |  | n261 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum CSI-RS Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

## B.2.5	Conditions for RRC connection release with redirection to NR

Editor’s notes for Table B.2.5.2-2:

- The value of Y for power classes 1 and 4 is FFS, where Y1 and Y4 are the rough/fine beam gain differences in Rx beam peak direction for power classes 1 and 4 respectively

- The value of Z for power classes 1 and 4 is FFS, where Z1 and Z4 are the rough/fine

This clause defines the following conditions for RRC connection release with redirection to NR: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.5-1 for FR1 NR cells.

The conditions are defined in Table B.2.5-2 for FR2 NR cells.

Table B.2.5-1: Conditions for RRC connection release with redirection to NR in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A | -125 | -122 | -4 |
|  | NR_FDD_FR1_B | -124.5 | -121.5 |  |
|  | NR_TDD_FR1_C | -124 | -121 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -124.5 | -120.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -123 | -120 |  |
|  | NR_FDD_FR1_G | -122 | -119 |  |
|  | NR_FDD_FR1_H | -121.5 | -118.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

Table B.2.5-2: Conditions for RRC connection release with redirection to NR in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE power class |  |  |  | UE power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -126.3+Y1 | -111.8 | -110.1 | -125.8+Y4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -126.3+Y1 | -111.8 | -110.1 | -125.8+Y4 |  |  |
|  |  | n259 |  |  | -106.5 |  |  |  |
|  |  | n260 | -123.3+Y1 |  | -107.5 | -123.8+Y4 |  |  |
|  |  | n261 | -126.3+Y1 | -111.8 | -110.1 | -125.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -118.3+Z1 | -100.8 | -99.2 | -116.8+Z4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -118.3+Z1 | -100.8 | -99.2 | -116.8+Z4 |  |  |
|  |  | n259 |  |  | -93.7 |  |  |  |
|  |  | n260 | -115.3+Z1 |  | -94.9 | -111.8+Z4 |  |  |
|  |  | n261 | -114.3 | -100.8 | -99.2 | -116.8+Z4 |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

## B.2.6	Conditions for UE transmit timing

### B.2.6.1	Conditions for SSB based UE transmit timing

Editor’s notes for Table B.2.6.1-2:

- The value of Y for power classes 1 and 4 is FFS, where Y1 and Y4 are the rough/fine beam gain differences in Rx beam peak direction for power classes 1 and 4 respectively

- The value of Z for power classes 1 and 4 is FFS, where Z1 and Z4 are the rough/fine beam gain differences in spherical coverage directions for power classes 1 and 4 respectively

This clause defines the following conditions for UE transmit timing adjustment performed based on SSBs: SSB_RP and SSB Ês/Iot and applicable for a corresponding operating band.

The conditions are defined in Table B.2.6.1-1 for FR1 SSB.

Table B.2.6.1-1: Conditions for SSB based UE transmit timing in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB=15 kHz | SCSSSB=30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -121 | -3 |
|  | NR_FDD_FR1_B | -123.5 | -120.5 |  |
|  | NR_TDD_FR1_C | -123 | -120 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 |  |
|  | NR_FDD_FR1_G | -121 | -118 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

The conditions are defined in Table B.2.6.1-2 for FR2 SSB.

Table B.2.6.1-2: Conditions for SSB based UE transmit timing in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE power class |  |  |  | UE power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-3 |
|  |  | n258 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 |  |  |
|  |  | n259 |  |  | -105.5 |  |  |  |
|  |  | n260 | -122.3+Y1 |  | -106.5 | -122.8+Y4 |  |  |
|  |  | n261 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-3 |
|  |  | n258 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 |  |  |
|  |  | n259 |  |  | -92.7 |  |  |  |
|  |  | n260 | -114.3+Z1 |  | -93.9 | -110.8+Z4 |  |  |
|  |  | n261 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOET 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

# B.3	RRM requirement exceptions

## B.3.1	Introduction

Annex B.3 covers exceptions for the side conditions based on receiver sensitivity for CA, DC, and SUL.

## B.3.2	Receiver sensitivity relaxation for CA

### B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1

For a UE supporting inter-band carrier aggregation configuration with uplink in NR band, if there is a relaxation of receiver sensitivity ΔRIB,c>0 dB as defined in TS 38.101-1 [18, Clause 7.3A.3], the relevant side conditions specifying received power levels (SSB_RP and Io) shall be increased by the amount Δ=ΔRIB,c defined for the corresponding downlink NR bands.

For a UE supporting CA configuration in FR1, the requirement in this clause applies for both SC and CA operation.

### B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1

#### B.3.2.2.1	Inter-band carrier aggregation

For a UE configured with inter-band carrier aggregation with active uplink in NR band, if there is a relaxation of receiver sensitivity ΔRIB,c>0 dB as defined in TS 38.101-1, Clause 7.3A.3 [2], the relevant side conditions specifying received power levels (SSB_RP and Io) shall be increased by the amount Δ=ΔRIB,c defined for the corresponding downlink NR bands.

If the relaxation Δ specified in this clause applies, then the relaxation specified in Clause B.3.2.1 should not be applied.

#### B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA

In this clause, requirements exceptions are described for the UE configured with a band in FR1 when it is impacted by UL harmonic interference from another band in FR1 of the same CA configuration.

A relevant side condition (SSB_RP and Io) in a requirement shall be increased by the amount Δ=L2-L1, where L1 is the reference sensitivity level specified in TS 38.101-1 [2], Clause 7.3.2, and L2 is the reference sensitivity level based on the requirements in TS 38.101-1 [2], Clause 7.3A.4, when the following conditions are fulfilled,

-	corresponding downlink component carriers on different NR bands are configured with CA and active,

-	the uplink is configured in the NR low operating band and is active,

-	the uplink configuration is as specified in TS 38.101-1 [2], Clause 7.3A.4, and

-	the exception requirements specified in TS 38.101-1 [2], Clause 7.3A.4 apply.

If the relaxation Δ specified in this clause applies, then the relaxation specified in Clause B.3.2.1 should not be applied.

#### B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA

In this clause, requirements exceptions are described for the UE with an inter-band carrier aggregation with uplink assigned to two NR bands.

A relevant side condition (SSB_RP and Io) in a requirement shall be increased by the amount Δ=L2-L1, where L1 is the reference sensitivity level specified in TS 38.101-1 [2], Clause 7.3.2, and L2 is the reference sensitivity level based on the requirements in TS 38.101-1 [2], Clause 7.3A.5, when the following conditions are fulfilled,

-	corresponding downlink component carriers on different bands are configured with CA and active,

-	uplinks are assigned to two NR bands,

-	the exception requirements specified in TS 38.101-1 [2], Clause 7.3A.5 apply.

If the relaxation Δ specified in this clause applies, then the relaxation specified in Clause B.3.2.1 should not be applied.

### B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2

Editor’s note: TBD

### B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2

#### B.3.2.4.1	Intra-band contiguous carrier aggregation

For a UE configured with intra-band contiguous carrier aggregation in NR band in FR2, if there is a relaxation of receiver sensitivity ΔRIB>0 dB as defined in TS 38.101-2 [3], Clause 7.3A.2.1 depending on the aggregated channel bandwidth, the relevant side conditions specifying received power levels (SSB_RP and Io) shall be increased by the amount Δ=ΔRIB defined for the corresponding downlink NR bands.

#### B.3.2.4.2	Intra-band non-contiguous carrier aggregation

For a UE configured with intra-band non-contiguous carrier aggregation in NR band in FR2, if there is a relaxation of receiver sensitivity ΔRIB>0 dB as defined in TS 38.101-2 [3], Clause 7.3A.2.1 depending on the aggregated channel bandwidth, the relevant side conditions specifying received power levels (SSB_RP and Io) shall be increased by the amount Δ=ΔRIB defined for the corresponding downlink NR bands.

## B.3.3	Receiver sensitivity relaxation for DC

Editor’s note: TBD

## B.3.4	Receiver sensitivity relaxation for SUL

### B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1

For a UE supporting a SUL configuration in FR1, if there is a relaxation of receiver sensitivity ΔRIB,c>0 dB as defined in TS 38.101-1 [2], Clause 7.3C.3, the relevant side conditions specifying received power levels (SSB_RP and Io) shall be increased by the amount Δ=ΔRIB,c defined for the corresponding downlink NR bands.

For a UE supporting a SUL configuration in FR1, the requirement in this clause applies for both SC and SUL operation.

### B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1

#### B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL

In this clause, requirements exceptions are described for the UE with a band in FR1 when it is impacted by UL harmonic interference from another band in FR1 of the same SUL configuration.

A relevant side condition (SSB_RP and Io) in a requirement shall be increased by the amount Δ=L2-L1, where L1 is the reference sensitivity level specified in TS 38.101-1 [2], Clause 7.3.2, and L2 is the reference sensitivity level based on the requirements in TS 38.101-1 [2], Clause 7.3C.2, when the following conditions are fulfilled,

-	a downlink component carrier is configured in NR band and is active,

-	the uplink is configured in the NR low operating band and is active,

-	the uplink configuration is as specified in TS 38.101-1 [2], Clause 7.3C.2, and

-	the exception requirements specified in TS 38.101-1 [2], Clause 7.3C.2 apply.

If the relaxation Δ specified in this clause applies, then the relaxation specified in Clause B.3.4.1 should not be applied.

Annex C (normative):
Downlink physical channels and propagation conditions

# C.1	Downlink physical channels

The following clauses describe the downlink physical channels that are needed for setting a connection and channels that are needed during a connection.

## C.1.1	General

TBD

## C.1.2	Default downlink signal levels

The downlink power settings in Table C.1.2-1 and Table C.1.2-2 are used unless otherwise specified in a test case for FR1 cell and FR2 cell, respectively. The downlink power settings in Table C.1.2-1 and Table C.1.2-2 are also used for the initial registration for NR SA test cases in clauses 6 and 7. For EN-DC test cases in clauses 4 and 5, the E-UTRA power settings used for initial registration are defined in Annex A.6.

For FR1, if the UE has more than one Rx antenna, the downlink signal is applied to each one. All UE Rx antennas shall be connected.

Table C.1.2-1: Default Downlink power levels for NR (FR1)

| SCS (kHz) | Parameter | Unit | Value |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Channel bandwidth | MHz | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 |
| 15 | Number of RBs |  | 25 | 50 | 75 | 100 | 128 | 160 | 215 | 270 | N/A | N/A | N/A | N/A |
|  | Channel BW power | dBm | -60 | -57 | -55 | -54 | -53 | -52 | -51 | -50 | N/A | N/A | N/A | N/A |
| 30 | Number of RBs |  | 10 | 24 | 36 | 50 | 64 | 75 | 100 | 128 | 162 | 216 | 243 | 270 |
|  | Channel BW power | dBm | -61 | -57 | -55 | -54 | -53 | -52 | -51 | -50 | -49 | -48 | -47 | -47 |
| 60 | Number of RBs |  | N/A | 10 | 18 | 24 | 30 | 36 | 50 | 64 | 75 | 100 | 120 | 135 |
|  | Channel BW power | dBm | N/A | -58 | -56 | -54 | -53 | -52 | -51 | -50 | -49 | -48 | -47 | -47 |
|  | RS EPRE | dBm/ 15kHz | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 |
| NOTE 1: The channel bandwidth powers are informative, based on -85dBm/15kHz SS/PBCH SSS EPRE, then scaled according to the number of RBs and rounded to the nearest integer dBm value. Full RE allocation with no boost or deboost is assumed. NOTE 2: The power level is specified at each UE Rx antenna. NOTE 3: DL level is applied for any of the Subcarrier Spacing configuration ( ) with the same power spectrum density of -85dBm/15kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table C.1.2-2: Default Downlink power levels for NR (FR2)

| SCS (kHz) |  | Unit | Channel Bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| 60 | Number of RBs |  | 66 | 132 | 264 | N/A |
|  | Channel BW power | dBm | -70 | -67 | -64 | N/A |
| 120 | Number of RBs |  | 32 | 66 | 132 | 264 |
|  | Channel BW power | dBm | -70 | -67 | -64 | -61 |
|  | SS/PBCH SSS EPRE | dBm/60kHz | -99 | -99 | -99 | -99 |
| NOTE 1: The channel bandwidth powers are informative, based on -99dBm/60kHz SS/PBCH SSS EPRE, then scaled according to the number of RBs and rounded to the nearest integer dBm value. Full RE allocation with no boost or deboost is assumed. NOTE 2: The power level is specified at the centre of quiet zone. NOTE 3: DL level is applied for any of the Subcarrier Spacing configuration () with the same power spectrum density of –99dBm/60kHz. |  |  |  |  |  |  |

The default signal level uncertainty for FR1 is +/-3dB at each test port, for any level specified. The default signal level uncertainty for FR2 is +/-6dB at center of quiet zone, for any level specified. If the uncertainty value is critical for the Test purpose, a tighter uncertainty is specified for the related test case in Annex F

## C.1.3	Default connection setup

Table C.1.3-1 describes the downlink physical channels that are required for NR connection setup. For EN-DC test cases in clauses 4 and 5, the required E-UTRA downlink physical channels are defined in TS 36.521-3 [26] Annex C.2.

Table C.1.3-1: Downlink physical channels required for NR connection set-up

| Physical Channel | EPRE Ratio | Note |
| --- | --- | --- |
| PBCH | PBCH_RA = 0 dB |  |
|  | PBCH_RB = 0 dB |  |
| PSS | PSS_RA = 0 dB |  |
| SSS | SSS_RA = 0 dB |  |
| PDCCH | PDCCH_RA = 0 dB |  |
|  | PDCCH_RB = 0 dB |  |
|  | MPDCCH_RB = 0 dB |  |
| PDSCH | PDSCH_RA = 0 dB |  |
|  | PDSCH_RB = 0 dB |  |
| DMRS | TBD |  |
| CSI-RS | TBD |  |
| NOTE 1: No boosting is applied |  |  |

As common PDSCH and PDCCH configuration parameters the parameters in Table A.1 and C.1.3-2 shall be used to bring up the connection setup for FR1 and FR2 NR cell.

Table C.1.3-2: PDSCH and PDCCH configuration

| Parameter | Unit | Value |
| --- | --- | --- |
| Number of HARQ processes |  | 8 (TDD) 4 (FDD) |
| Maximum number of HARQ transmission |  | 5 |

# C.2	Propagation conditions

The propagation conditions and channel models for various environments are specified. For each environment a propagation model is used to evaluate the propagation pathless due to the distance. Channel models are formed by combining delay profiles with a Doppler spectrum, with the addition of correlation properties in the case of a multi-antenna scenario.

## C.2.1	No interference

The downlink connection between the SS and the UE is without AWGN, and has no fading or multipath effects.

## C.2.2	Static propagation conditions

The downlink connection between the SS and the UE is an AWGN environment (unless otherwise stated) with no fading or multipath effects.

### C.2.2.1	UE receiver with 2Rx antenna connectors

For 1 port transmission to UE receiver with 2Rx the channel matrix is defined in the frequency domain by

.

For 2 port transmission to UE Receiver with 2Rx the channel matrix is defined in the frequency domain by

For 4 port transmission to UE Receiver with 2Rx the channel matrix is defined in the frequency domain by

### C.2.2.2	UE receiver with 4Rx antenna connectors

For 1 port transmission to UE receiver with 4Rx the channel matrix is defined in the frequency domain by

.

For 2 port transmission to UE Receiver with 4Rx the channel matrix is defined in the frequency domain by

.

For 4 port transmission to UE Receiver with 4Rx the channel matrix is defined in the frequency domain by

.

## C.2.3	Multi-path fading propagation conditions

Same as defined in TS 38.521-4 [20] clause B.2.

Annex D (normative):
Deviations from standard test configuration

This annex summarizes the list of test cases which deviate from the standard test configuration.

# D.1	Test cases with different numerologies

TBD

# D.2	EN-DC test cases with different EN-DC configurations

In clauses 4 and 5, EN-DC test cases may be defined for two component carriers (CCs) as well as for more than two CCs to verify the same RRM requirement.

## D.2.1	Principle of testing

If multiple EN-DC test cases are defined for two CCs as well as for more than two CCs to verify the same type of RRM requirement, and this requirement is dependent on the number of CCs, then from the UE performance point of view the test coverage can be considered fulfilled by executing only the EN-DC test cases with the maximum number of CCs in EN-DC supported by the UE. Otherwise if the same type of RRM requirement is independent of the number of CCs then from the UE performance point of view the test coverage can be considered fulfilled by executing only the EN-DC test cases with two CCs in EN-DC supported by the UE.

Editor’s Note: The maximum number of CCs that can be used in FR2 tests in EN-DC would depend on the test equipment capability.

# D.3	Carrier aggregation test cases with different CA configurations

In clauses 6 and 7, carrier aggregation test cases may be defined for two CCs as well as for more than two CCs to verify the same RRM requirement.

## D.3.1	Principle of testing

If multiple carrier aggregation test cases are defined for two CCs as well as for more than two CCs to verify the same RRM requirement, and the test requirement is dependent on the number of CCs, then from the UE performance point of view the test coverage can be considered fulfilled by executing only the CA test cases with the maximum number of CCs in CA supported by the UE. Otherwise if the same type of RRM requirement is independent of the number of CCs then from the UE performance point of view the test coverage can be considered fulfilled by executing only the CA test cases with at least two CCs in CA supported by the UE.

Editor’s Note: The maximum number of CCs that can be used in FR2 tests in CA would depend on the test equipment capability.

# D.4	Antenna connection for 4Rx capable UEs

All the tests in this test specification are defined for UEs supporting 2Rx. This clause explains how to apply the 2Rx tests in clauses 4 and 6 to UEs supporting 4Rx antenna ports. No tests are currently specified in clauses 4 or 6 which are applicable only to 4Rx antenna ports, so 4Rx capable UEs are always tested by reusing tests which were originally specified for 2Rx UEs. Please notice that 4Rx is in general not supported for the test cases in clauses 5 and 7.

## D.4.1	Principle of testing

### D.4.1.1	Single carrier tests

For 4Rx capable UEs supporting at least one band where 2Rx is supported and 4Rx is not supported, all single carrier tests specified in clauses 4 and 6, except 4.7 and 6.7 shall be tested with 2Rx on any band where 2Rx is supported and 4Rx is not supported, with the antenna connection defined in D.4.2.1. Single carrier tests specified clauses 4.7 and 6.7 are band dependent and shall be tested in all bands supported by the UE, using 2Rx and the antenna connection defined in D.4.2.1 for the bands where 2Rx is supported and 4Rx is not supported, and the antenna connection defined in D.4.2.2 for the bands where 2Rx is not supported.

For 4Rx capable UEs that do not support any band where 2Rx is supported and 4Rx is not supported, all single carrier tests in clauses 4 and 6 shall be tested with 4Rx using the antenna configuration defined in D.4.2.2.For radio link monitoring tests, the SNR levels are modified according to table D.4.1.1-1 and table D.4.1.1-2

Table D.4.1.1-1: Modified parameters for RLM out of sync testing with 4 RX antenna connection

| Test case | SNR during T3 (dB) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Test 1 | Test 2 | Test 3 | Test 4 |
| 4.5.1.1 | -18 | N/A | N/A | N/A |
| 4.5.1.3 | -18 | N/A | N/A | N/A |
| 4.5.1.5 | -18 | N/A | N/A | N/A |
| 4.5.1.7 | -18 | N/A | N/A | N/A |
| 5.5.1.1 | -18 | N/A | N/A | N/A |
| 5.5.1.3 | -18 | N/A | N/A | N/A |
| 5.5.1.5 | -18 | N/A | N/A | N/A |
| 5.5.1.7 | -18 | N/A | N/A | N/A |
| 6.5.1.1 | -18 | N/A | N/A | N/A |
| 6.5.1.3 | -18 | N/A | N/A | N/A |
| 6.5.1.5 | -18 | N/A | N/A | N/A |
| 6.5.1.7 | -18 | N/A | N/A | N/A |
| 7.5.1.1 | -18 | N/A | N/A | N/A |
| 7.5.1.3 | -18 | N/A | N/A | N/A |
| 7.5.1.5 | -18 | N/A | N/A | N/A |
| 7.5.1.7 | -18 | N/A | N/A | N/A |

Table D.4.1.1-2: Modified parameters for RLM in sync single carrier testing with 4 RX antenna connection

| Test case | SNR during T3 (dB) |  | SNR during T4 (dB) |  |
| --- | --- | --- | --- | --- |
|  | Test 1 | Test 2 | Test 1 | Test 2 |
| 4.5.1.2 | -18 | N/A | -8 | N/A |
| 4.5.1.4 | -18 | N/A | -8 | N/A |
| 4.5.1.6 | -18 | N/A | -8 | N/A |
| 4.5.1.8 | -18 | N/A | -8 | N/A |
| 5.5.1.2 | -18 | N/A | -8 | N/A |
| 5.5.1.4 | -18 | N/A | -8 | N/A |
| 5.5.1.6 | -18 | N/A | -8 | N/A |
| 5.5.1.8 | -18 | N/A | -8 | N/A |
| 6.5.1.2 | -18 | N/A | -8 | N/A |
| 6.5.1.4 | -18 | N/A | -8 | N/A |
| 6.5.1.6 | -18 | N/A | -8 | N/A |
| 6.5.1.8 | -18 | N/A | -8 | N/A |
| 7.5.1.2 | -18 | N/A | -8 | N/A |
| 7.5.1.4 | -18 | N/A | -8 | N/A |
| 7.5.1.6 | -18 | N/A | -8 | N/A |
| 7.5.1.8 | -18 | N/A | -8 | N/A |

Table D.4.1.1-3: Modified parameters for Beam Failure Detection and Link Recovery testing with 4 RX antenna connection

| Test case | SNR for RS in set q0 during T3, T4 and T5.(dB) |
| --- | --- |
|  | Test 1 |
| 4.5.5.1 | -15 |
| 4.5.5.2 | -15 |
| 4.5.5.3 | -15 |
| 4.5.5.4 | -15 |
| 5.5.5.1 | -15 |
| 5.5.5.2 | -15 |
| 5.5.5.3 | -15 |
| 5.5.5.4 | -15 |
| 6.5.5.1 | -15 |
| 6.5.5.2 | -15 |
| 6.5.5.3 | -15 |
| 6.5.5.4 | -15 |
| 7.5.5.1 | -15 |
| 7.5.5.2 | -15 |
| 7.5.5.3 | -15 |
| 7.5.5.4 | -15 |

### D.4.1.2	Carrier aggregation tests

For carrier aggregation tests, the antenna connection is selected independently for each cell, the PCell and the SCell(s). If a cell (either PCell or any of the SCell(s)) is on a band where 2Rx is supported and 4Rx is not supported, antenna connection in Clause D.4.2.1 shall be used for this cell. If the cell is on a band where 2Rx is not supported, antenna connection in clause D.4.2.2 shall be used for this cell.

### D.4.1.3	EN-DC tests

For all EN-DC tests, the antenna connection is selected independently for each cell. For the E-UTRA PCell, the antenna connection specified in D.4.2.3 shall be used if the PCell is on an E-UTRA band where 2Rx is supported and 4Rx is not supported, and the antenna connection specified in D.4.2.4 shall be used if the PCell is on an E-UTRA band not supporting 2Rx.

For the NR PSCell and SCell(s), the principle of testing is the same as in D.4.1.2.

## D.4.2	Antenna connection

### D.4.2.1	Antenna connection for NR bands where 2Rx is supported

For NR bands where 2Rx is supported and 4Rx is not supported, the UE shall decide via manufacturer declaration and antenna port configuration which 2 of the 4 antenna ports shall be connected with the downlink signal from the SS. The remaining 2 antenna ports shall be connected to zero input. The parameters and test requirements remain unmodified.

### D.4.2.2	Antenna connection for NR bands where only 4Rx is supported

For NR bands where only 4Rx is supported, all 4Rx antenna ports shall be connected to the downlink signal from the SS. The system simulator shall provide independent noise and fading (low correlation) for each antenna port. Except for the modifications to radio link monitoring tests specified in D.4.1.1 and D.4.1.2, the parameters and test requirements remain unmodified.

### D.4.2.3	Antenna connection for E-UTRA bands where 2Rx is supported

For E-UTRAN bands where 2Rx is supported and 4Rx is not supported, the UE shall decide via manufacturer declaration and antenna port configuration which 2 of the 4 antenna ports shall be connected with the downlink signal from the SS. The remaining 2Rx antenna ports shall be connected to zero input. The parameters and test requirements remain unmodified.

### D.4.2.4	Antenna connection for E-UTRA bands where only 4Rx is supported

For E-UTRAN bands where only 4Rx is supported, all 4Rx antenna ports shall be connected to the downlink signal from the SS. The system simulator shall provide independent noise and fading (low correlation) for each antenna port. Except for the modifications to radio link monitoring tests specified in TS36.521-1 [26] clause 3A.4.1.2.1 and 3A.4.1.2.2, the parameters and test requirements remain unmodified.

# D.5	Test Cases with Different Channel Bandwidths

## D.5.1	Test Cases with Different E-UTRA Channel Bandwidths

### D.5.1.1	Introduction

Test cases involving E-UTRA cell(s) may be defined with different E-UTRA channel bandwidths to verify the same type of RRM requirement.

### D.5.1.2	Principle of testing

If multiple test cases involving E-UTRA cell(s) are defined with different E-UTRA channel bandwidths to verify the same type of RRM requirement that is E-UTRA channel bandwidth independent, then the UE needs to be tested with only one channel bandwidth in each E-UTRA cell and with the same bandwidth in all the E-UTRA cells used in the test case.

# D.6	Test Cases for Synchronous and Asynchronous DC Operations

## D.6.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations

### D.6.1.1	Introduction

This clause defines a principle, which is applicable to test cases verifying RRM requirements for EN-DC operation in synchronous and asynchronous scenarios.

Test cases may be defined in both synchronous EN-DC and asynchronous EN-DC scenarios to verify the same type of RRM requirement.

### D.6.1.2	Principle of Testing

If EN-DC test cases are defined in both synchronous and asynchronous EN-DC scenarios to verify the same type of RRM requirement then the UE capable of both synchronous and asynchronous EN-DC operations needs to be tested with one of the tests in either synchronous or asynchronous EN-DC scenarios.

Annex E (normative):
Cell configuration mapping

The cells used in TS 38.533 do not correspond to the cells defined in TS 38.508-1 [14] clause 4.4.2. This annex describes the mapping between the test cases in TS 38.533 and the cells defined in TS 38.508-1 [14]. The test case shall apply the RF parameters as defined in TS 38.533 according to the column heading.

NOTE:	For example if the second cell in a test case is an inter-frequency cell then Cell3 from TS 38.508-1 [14] clause 4.4.2 is used with the radio parameters as defined for Cell2 in TS 38.533.

# E.1	Test frequency selection

The requirements defined in this test specification comprise EN-DC and NR SA test cases. The test cases are defined with a single NR cell or with multiple NR cells. The multi-cell test cases can be either intra-frequency, i.e. the NR cells defined by the test are overlapping in the frequency domain, or inter-frequency, i.e. the NR cells defined by the test have different centre frequencies, separated from each other by a frequency value bigger than the respective cell bandwidths. This clause describes the general rule on how to select the test frequencies for the NR RRM test cases in this test specification.

## E.1.1	E-UTRA PCell for EN-DC test cases

Unless otherwise stated, the E-UTRA PCell for EN-DC test cases shall be configured using the test frequency “Mid” as defined in TS 36.508 [25] for the corresponding E-UTRA band.

In case that the “Mid” test frequency overlaps with any of the NR test frequencies required by the test case, the E-UTRA PCell shall be shifted to an additional frequency within the E-UTRA same band. If the E-UTRA band channel bandwidth is not sufficient to allocate a non-overlapping E-UTRA PCell, the auxiliary band as defined in TS 36.521-3 [26] clause 3 shall be used.

## E.1.2	Test cases with one NR cell

Unless otherwise stated, for NR test cases with one NR cell, this cell shall be configured using the test frequency “Mid” as defined in TS 38.508-1 [14] for the corresponding band under test.

## E.1.3	Test cases with more than one NR cell

### E.1.3.1	Intra-frequency test cases

Unless otherwise stated, multi-cell intra-frequency test cases shall be tested using the test frequency “Mid” as defined in TS 38.508-1 [14] for the corresponding NR band under test.

### E.1.3.2	Inter-frequency test cases

For NR SA  and EN-DC multi-cell inter-frequency test cases in FR1, unless otherwise stated, the serving cell shall be configured using the test frequency “Low” as defined in TS 38.508-1 [14] for the corresponding band under test. Any inter-frequency neighbour cell shall be configured using the test frequency “High” as defined in TS 38.508-1 [14] for the corresponding band under test.

For NR SA multi-cell inter-frequency test cases in FR2, unless otherwise stated, the serving cell (and any other neighbour cell in the same frequency carrier) shall be configured using the test frequency “Mid” as defined in TS 38.508-1 [14], clause 7.2.3.2 for the corresponding band under test. Any inter-frequency neighbour cell shall be configured using a non-overlapping test frequency adjacent to the serving cell frequency, as defined in TS 38.508-1 [14], clause 7.2.3.2.

For EN-DC multi-cell inter-frequency test cases in FR2, unless otherwise stated, the PSCell (and any other neighbour cell in the same frequency carrier) shall be configured using the test frequency “Mid” as defined in TS 38.508-1 [14] , clause 7.2.3.2 for the corresponding band under test. Any inter-frequency neighbour cell shall be configured using a non-overlapping test frequency adjacent to the PSCell frequency, as defined in TS 38.508-1 [14] , clause 7.2.3.2.

## E.1.4	Carrier aggregation test cases

### E.1.4.1	Inter-band carrier aggregation

For inter-band carrier aggregation test cases, each of the component carriers and their respective neighbour cells shall be configured following the sample principles defined in E.1.2 and E.1.3.

### E.1.4.2	Intra-band contiguous carrier aggregation

For intra-band contiguous carrier aggregation, the test frequency selection shall be done following the same principle as in E.1.3.2 for inter-frequency test cases.

### E.1.4.3	Intra-band non-contiguous carrier aggregation

For intra-band non-contiguous carrier aggregation in FR1, unless otherwise specified, the test frequency selection shall be done following the maximum Wgap principle, i.e. selecting the test frequencies (of the test frequencies defined in TS 38.508-1 [14]) with the widest frequency separation within the band under test.

For intra-band non-contiguous carrier aggregation in FR2, the test frequency selection is TBD.

## E.1.5	E-UTRA – NR inter RAT test cases

Unless otherwise stated, the E-UTRA serving/neighbour cell for E-UTRA – NR inter-RAT test cases shall be configured using the test frequency “Mid” as defined in TS 36.508 [25] for the corresponding E-UTRA band.

In case that the “Mid” test frequency overlaps with any of the NR test frequencies required by the test case, the E-UTRA cell shall be shifted to an additional frequency within the E-UTRA same band. If the E-UTRA band channel bandwidth is not sufficient to allocate a non-overlapping E-UTRA cell, the auxiliary band as defined in TS 36.521-3 [26] clause 3 shall be used.

## E.1.6	Intra-band EN-DC test cases

### E.1.6.1	Intra-band non-contiguous EN-DC

For intra-band non-contiguous EN-DC in FR1, unless otherwise specified, the test frequency selection shall be done following the maximum Wgap principle, i.e. selecting the test frequencies (of the test frequencies defined in TS 38.508-1 [14]) with the widest frequency separation within the band under test.

#### E.1.6.1.1	Inter frequency neighbour cell

Unless otherwise stated, any inter-frequency neighbour cell shall be configured using the test frequency “Mid” as defined in TS 38.508-1 [14] for the corresponding NR band under test. If the NR band channel bandwidth is not sufficient to allocate a non-overlapping the neighbour cell, the auxiliary band as defined in TBD shall be used.

### E.1.6.2	Intra-band contiguous EN-DC

#### E.1.6.1.1	E-UTRA PCell

Unless otherwise stated, the E-UTRA PCell for intra-band contiguous EN-DC test cases shall be configured using same test frequency, e.g., “Mid” as for NR PSCell as defined in TS 38.508-1 [14].

# E.2	Cell configuration mapping for EN-DC FR1 test cases in Chapter 4

Table E.2-1 defines the cell configuration mapping for EN-DC FR1 test cases in chapter 4 of this test specification.

Table E.2-1: Cell configuration mapping for EN-DC FR1 RRM testing

| TC | Description | 38.533 LTE Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 4.3.2.2.1 | EN-DC FR1 contention based random access | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.3.2.2.2 | EN-DC FR1 non-contention based random access | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.3.2.2.3 | EN-DC FR1 2-step contention based random access | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.3.2.2.4 | EN-DC FR1 2-step non-contention based random access | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.4.1.1 | EN-DC FR1 UE transmit timing accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.4.3.1 | EN-DC FR1 timing advance adjustment accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.1.1 | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.1.2 | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.1.3 | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.1.4 | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.1.5 | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.1.6 | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.1.7 | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.1.8 | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.2.1 | EN-DC FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.2.2 | EN-DC FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.2.3 | EN-DC FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | intra-band |
|  |  |  |  | NR Cell 10 |  | inter-band |
| 4.5.2.4 | EN-DC FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | intra-band |
|  |  |  |  | NR Cell 10 |  | inter-band |
| 4.5.2.5 | EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC | LTE Cell 6 | NR Cell 1 | LTE Cell 3 |  |  |
| 4.5.2.6 | EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC | LTE Cell 6 | NR Cell 1 | LTE Cell 3 |  |  |
| 4.5.3.1 | EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | intra-band |
|  |  |  |  | NR Cell 10 |  | inter-band |
| 4.5.3.2 | EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 320ms SCell measurement cycle | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | intra-band |
|  |  |  |  | NR Cell 10 |  | inter-band |
| 4.5.3.3 | EN-DC FR1 SCell activation and deactivation of unknown SCell in non-DRX | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | intra-band |
|  |  |  |  | NR Cell 10 |  | inter-band |
| 4.5.4.1 | EN-DC FR1 UE UL carrier RRC reconfiguration delay | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.5.5.1 | EN-DC FR1 SSB-based beam failure detection and link recovery in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.5.2 | EN-DC FR1 SSB-based beam failure detection and link recovery in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.5.3 | EN-DC FR1 CSI-RS-based beam failure detection and link recovery in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.5.4 | EN-DC FR1 CSI-RS-based beam failure detection and link recovery in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.5.5 | EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.5.6 | EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.6.1.1 | EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.6.1.2 | EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.5.6.2.1 | EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.7.1 | EN-DC FR1 addition and release delay of known PSCell | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.8.1 | EN-DC FR1 interruptions at switching between two uplink carriers | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.6.1.1 | EN-DC FR1 event-triggered reporting without gap in non-DRX | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 4.6.1.2 | EN-DC FR1 event-triggered reporting without gap in DRX | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 4.6.1.3 | EN-DC FR1 event-triggered reporting with gap in non-DRX | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 4.6.1.4 | EN-DC FR1 event-triggered reporting with gap in DRX | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 4.6.1.5 | EN-DC FR1 event-triggered reporting without gap in non-DRX with SSB time index detection | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 4.6.1.6 | EN-DC FR1 event-triggered reporting with gap in non-DRX with SSB time index detection | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 4.6.1.7 | EN-DC FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 4.6.2.1 | EN-DC FR1-FR1 event-triggered reporting in non-DRX | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.6.2.2 | EN-DC FR1-FR1 event-triggered reporting in DRX | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.6.2.5 | EN-DC FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.6.2.6 | EN-DC FR1-FR1 event-triggered reporting in DRX with SSB time index detection | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.6.4.1 | EN-DC FR1 SSB-based L1-RSRP measurement in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.6.4.2 | EN-DC FR1 SSB-based L1-RSRP measurement in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.6.4.3 | EN-DC FR1 CSI-RS-based L1-RSRP measurement in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.6.4.4 | EN-DC FR1 CSI-RS-based L1-RSRP measurement in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.7.1.1.1 | EN-DC FR1 SS-RSRP absolute measurement accuracy | LTE Cell 1 | NR Cell 489 | NR Cell 1 |  |  |
| 4.7.1.1.2 | EN-DC FR1 SS-RSRP relative measurement accuracy | LTE Cell 1 | NR Cell 489 | NR Cell 1 |  |  |
| 4.7.1.2.1 | EN-DC FR1-FR1 SS-RSRP absolute measurement accuracy | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.7.1.2.2 | EN-DC FR1-FR1 SS-RSRP relative measurement accuracy | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.7.2.1 | EN-DC FR1 SS-RSRQ measurement accuracy | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 4.7.2.2.1 | EN-DC FR1-FR1 SS-RSRQ absolute measurement accuracy | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.7.2.2.2 | EN-DC FR1-FR1 SS-RSRQ relative measurement accuracy | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.7.3.1 | EN-DC FR1 SS-SINR measurement accuracy | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 4.7.3.2.1 | EN-DC FR1-FR1 SS-SINR absolute measurement accuracy | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.7.3.2.2 | EN-DC FR1-FR1 SS-SINR relative measurement accuracy | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.7.4.1.1 | EN-DC FR1 SSB based L1-RSRP absolute measurement accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.7.4.1.2 | EN-DC FR1 SSB based L1-RSRP relative measurement accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.7.4.2.1 | EN-DC FR1 CSI-RS based L1-RSRP absolute measurement accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.7.4.2.2 | EN-DC FR1 CSI-RS based L1-RSRP relative measurement accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.7.5.1 | EN-DC FR1 SFTD measurement accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |

# E.3	Cell configuration mapping for EN-DC FR2 test cases in Chapter 5

Table E.3-1 defines the cell configuration mapping for EN-DC FR2 test cases in chapter 5 of this test specification.

Table E.3-1: Cell configuration mapping for EN-DC FR2 RRM testing

| TC | Description | 38.533 LTE Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 5.3.2.2.1 | EN-DC FR2 contention based random access | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.3.2.2.2 | EN-DC FR2 non-contention based random access | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.3.2.2.3 | EN-DC FR2 2-step contention based random access | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.3.2.2.4 | EN-DC FR2 2-step non-contention based random access | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.4.1.1 | EN-DC FR2 UE transmit timing accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.4.3.1 | EN-DC FR2 timing advance adjustment accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.1.1 | EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | LTE Cell 1 | NR Cell 6 |  |  |  |
| 5.5.1.3 | EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode | LTE Cell 1 | NR Cell 6 |  |  |  |
| 5.5.1.5 | EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.1.6 | EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.1.7 | EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.1.8 | EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.1.9 | EN-DC FR2 radio link monitoring UE scheduling restrictions | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.2.1 | EN-DC FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.2.2 | EN-DC FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.2.3 | EN-DC FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | intra-band |
|  |  |  |  | NR Cell 10 |  | inter-band |
| 5.5.2.4 | EN-DC FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | intra-band |
|  |  |  |  | NR Cell 10 |  | inter-band |
| 5.5.2.5 | EN-DC FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC | LTE Cell 6 | NR Cell 1 | LTE Cell 3 |  |  |
| 5.5.2.6 | EN-DC FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC | LTE Cell 6 | NR Cell 1 | LTE Cell 3 |  |  |
| 5.5.3.1 | EN-DC FR2 SCell activation and deactivation intra-band in non-DRX | LTE Cell 1 | NR Cell 1 | NR Cell 6 |  | intra-band |
| 5.5.5.1 | EN-DC FR2 SSB-based beam failure detection and link recovery in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.5.2 | EN-DC FR2 SSB-based beam failure detection and link recovery in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.5.3 | EN-DC FR2 CSI-RS-based beam failure detection and link recovery in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.5.4 | EN-DC FR2 CSI-RS-based beam failure detection and link recovery in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.5.5 | EN-DC FR2 scheduling available restriction during SSB-based beam failure detection and link recovery in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.8.1 | EN-DC FR2 MAC-CE based active TCI state switch | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.8.2 | EN-DC FR2 RRC based active TCI state switch | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.6.1.1 | EN-DC FR2 event-triggered reporting without gap in non-DRX | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 5.6.1.2 | EN-DC FR2 event-triggered reporting without gap in DRX | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 5.6.1.3 | EN-DC FR2 event-triggered reporting with gap in non-DRX | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 5.6.1.4 | EN-DC FR2 event-triggered reporting with gap in DRX | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 5.6.2.1 | EN-DC FR2-FR2 event-triggered reporting in non-DRX | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.6.2.2 | EN-DC FR2-FR2 event-triggered reporting in DRX | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.6.2.3 | EN-DC FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.6.2.4 | EN-DC FR2-FR2 event-triggered reporting in DRX with SSB time index detection | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.6.2.5 | EN-DC FR1-FR2 event-triggered reporting in non-DRX | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.6.2.6 | EN-DC FR1-FR2 event-triggered reporting in DRX | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.6.2.7 | EN-DC FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.6.2.8 | EN-DC FR1-FR2 event-triggered reporting in DRX with SSB time index detection | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.6.3.1 | EN-DC FR2 SSB-based L1-RSRP measurement in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.6.3.2 | EN-DC FR2 SSB-based L1-RSRP measurement in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.6.3.3 | EN-DC FR2 CSI-RS-based L1-RSRP measurement in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.6.3.4 | EN-DC FR2 CSI-RS-based L1-RSRP measurement in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.7.1.1 | EN-DC FR2 SS-RSRP measurement accuracy | LTE Cell 1 | NR Cell 489 | NR Cell 1 |  |  |
| 5.7.1.2 | EN-DC FR2-FR2 SS-RSRP measurement accuracy | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.7.2.1 | EN-DC FR2 SS-RSRQ measurement accuracy | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 5.7.2.2 | EN-DC FR2-FR2 SS-RSRQ measurement accuracy | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.7.3.1 | EN-DC FR2 SS-SINR measurement accuracy | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 5.7.3.2 | EN-DC FR2-FR2 SS-SINR measurement accuracy | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |

# E.4	Cell configuration mapping for SA FR1 test cases in Chapter 6

Table E.4-1 defines the cell configuration mapping for SA FR1 test cases in chapter 6 of this test specification.

Table E.4-1: Cell configuration mapping for SA FR1 RRM testing

| TC | Description | 38.533 NR Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6.1.1.1 | NR SA FR1 cell re-selection | NR Cell 1 | NR Cell 11 |  |  |  |  |
| 6.1.1.2 | NR SA FR1-FR1 cell re-selection | NR Cell 6 | NR Cell 23 |  |  |  |  |
| 6.1.1.3 | NR SA FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | NR Cell 1 | NR Cell 11 |  |  |  |  |
| 6.1.1.4 | NR SA FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | NR Cell 1 | NR Cell 11 |  |  |  |  |
| 6.1.1.5 | NR SA FR1-FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | NR Cell 1 | NR Cell 23 |  |  |  |  |
| 6.1.1.6 | NR SA FR1-FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | NR Cell 1 | NR Cell 23 |  |  |  |  |
| 6.1.1.7 | NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16 | NR Cell 1 | NR Cell 11 |  |  |  |  |
| 6.3.1.1 | NR SA FR1 handover with known target cell | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.3.1.2 | NR SA FR1 handover with unknown target cell | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.3.1.3 | NR SA FR1-FR1 Handover with unknown Target Cell | NR Cell 6 | NR Cell 3 |  |  |  |
| 6.3.1.7 | NR SA FR1 synchronous DAPS handover | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.3.1.8 | NR SA FR1 asynchronous DAPS handover | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.3.1.9 | NR SA FR1 Intra-band inter-frequency synchronous DAPS handover | NR Cell 6 | NR Cell 3 |  |  |  |
| 6.3.1.10 | NR SA FR1 Intra-band inter-frequency asynchronous DAPS handover | NR Cell 6 | NR Cell 3 |  |  |  |
| 6.3.1.11 | NR SA FR1 Inter-band inter-frequency synchronous DAPS handover | NR Cell 1 | NR Cell 10 |  |  |  |
| 6.3.1.12 | NR SA FR1 Inter-band inter-frequency asynchronous DAPS handover | NR Cell 1 | NR Cell 10 |  |  |  |
| 6.3.2.1.1 | NR SA FR1 RRC re-establishment | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.3.2.1.2 | NR SA FR1 - FR1 RRC re-establishment | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.3.2.1.3 | NR SA FR1 RRC re-establishment without serving cell timing | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.3.2.2.1 | NR SA FR1 contention based random access | NR Cell 1 |  |  |  |  |  |
| 6.3.2.2.2 | NR SA FR1 non-contention based random access | NR Cell 1 |  |  |  |  |  |
| 6.3.2.2.3 | NR SA FR1 2-step contention based random access | NR Cell 1 |  |  |  |  |  |
| 6.3.2.2.4 | NR SA FR1 2-step non-contention based random access | NR Cell 1 |  |  |  |  |  |
| 6.3.2.3.1 | NR SA FR1 RRC connection release with redirection | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.3.3.1 | NR SA FR1 conditional handover | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.3.3.2 | NR SA FR1-FR1 conditional handover | NR Cell 1 | NR Cell 6 |  |  |  |  |
| 6.4.3.1 | NR SA FR1 timing advance adjustment accuracy | NR Cell 1 |  |  |  |  |  |
| 6.5.1.1 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode | NR Cell 1 |  |  |  |  |  |
| 6.5.1.2 | NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode | NR Cell 1 |  |  |  |  |  |
| 6.5.1.3 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode | NR Cell 1 |  |  |  |  |  |
| 6.5.1.4 | NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode | NR Cell 1 |  |  |  |  |  |
| 6.5.1.5 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode | NR Cell 1 |  |  |  |  |  |
| 6.5.1.6 | NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode | NR Cell 1 |  |  |  |  |  |
| 6.5.1.7 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode | NR Cell 1 |  |  |  |  |  |
| 6.5.1.8 | NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode | NR Cell 1 |  |  |  |  |  |
| 6.5.2.1 | NR SA FR1 interruptions during measurements on deactivated NR SCC | NR Cell 1 | NR Cell 6 |  |  | intra-band |  |
|  |  |  | NR Cell 10 |  |  | inter-band |  |
| 6.5.3.1 | NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle | NR Cell 1 | NR Cell 6 |  |  | intra-band |  |
|  |  |  | NR Cell 10 |  |  | inter-band |  |
| 6.5.3.2 | NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 320ms SCell measurement cycle | NR Cell 1 | NR Cell 6 |  |  | intra-band |  |
|  |  |  | NR Cell 10 |  |  | inter-band |  |
| 6.5.3.3 | NR SA FR1 SCell activation and deactivation of unknown SCell in non-DRX | NR Cell 1 | NR Cell 6 |  |  | intra-band |  |
|  |  |  | NR Cell 10 |  |  | inter-band |  |
| 6.5.4.1 | NR SA FR1 UE UL carrier RRC reconfiguration delay | NR Cell 1 | NR Cell 33 |  |  |  |  |
| 6.5.5.1 | NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX | NR Cell 1 |  |  |  |  |  |
| 6.5.5.2 | NR SA FR1 SSB-based beam failure detection and link recovery in DRX | NR Cell 1 |  |  |  |  |  |
| 6.5.5.3 | NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX | NR Cell 1 |  |  |  |  |  |
| 6.5.5.4 | NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX | NR Cell 1 |  |  |  |  |  |
| 6.5.6.1.1 | NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.5.6.1.2 | NR SA FR1 DCI-based DL active BWP switch in non-DRX | NR Cell 1 |  |  |  |  |  |
| 6.5.6.2.1 | NR SA FR1 RRC-based DL active BWP switch in non-DRX | NR Cell 1 |  |  |  |  |  |
| 6.6.1.1 | SA event triggered reporting tests without gap under non-DRX | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.6.1.2 | SA event triggered reporting tests without gap under DRX | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.6.1.3 | SA event triggered reporting tests with per-UE gaps under non-DRX | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.6.1.4 | SA event triggered reporting tests with per-UE gaps under DRX | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.6.1.5 | SA event triggered reporting tests without gap under non-DRX with SSB index reading | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.6.1.6 | SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.6.1.7 | NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.6.2.1 | NR SA FR1-FR1 event-triggered reporting in non-DRX | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.6.2.2 | NR SA FR1-FR1 event-triggered reporting in DRX | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.6.2.5 | NR SA FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.6.2.6 | NR SA FR1-FR1 event-triggered reporting in DRX with SSB time index detection | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.6.4.1 | NR SA FR1 SSB-based L1-RSRP measurement in non-DRX | NR Cell 1 |  |  |  |  |  |
| 6.6.4.2 | NR SA FR1 SSB-based L1-RSRP measurement in DRX | NR Cell 1 |  |  |  |  |  |
| 6.6.4.3 | NR SA FR1 CSI-RS-based L1-RSRP measurement in non-DRX | NR Cell 1 |  |  |  |  |  |
| 6.6.4.4 | NR SA FR1 CSI-RS-based L1-RSRP measurement in DRX | NR Cell 1 |  |  |  |  |  |
| 6.6.4.5 | NR SA FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16 | NR Cell 1 |  |  |  |  |  |
| 6.7.1.3.1 | NR SA FR1-FR2 SS-RSRP absolute measurement accuracy | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.7.1.3.2 | NR SA FR1-FR2 SS-RSRP relative measurement accuracy | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.7.1.1.1 | NR SA FR1 SS-RSRP absolute measurement accuracy | NR Cell 489 | NR Cell 1 |  |  |  |  |
| 6.7.1.1.2 | NR SA FR1 SS-RSRP relative measurement accuracy | NR Cell 489 | NR Cell 1 |  |  |  |  |
| 6.7.1.2.1 | NR SA FR1-FR1 SS-RSRP absolute measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.7.1.2.2 | NR SA FR1-FR1 SS-RSRP relative measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.7.2.1 | NR SA FR1 SS-RSRQ measurement accuracy | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.7.2.2.1 | NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.7.2.2.2 | NR SA FR1-FR1 SS-RSRQ relative measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.7.3.1 | NR SA FR1 SS-SINR measurement accuracy | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.7.3.2.1 | NR SA FR1-FR1 SS-SINR absolute measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.7.3.2.2 | NR SA FR1-FR1 SS-SINR relative measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.7.4.1.1 | NR SA FR1 SSB based L1-RSRP absolute measurement accuracy | NR Cell 1 |  |  |  |  |  |
| 6.7.4.1.2 | NR SA FR1 SSB based L1-RSRP relative measurement accuracy | NR Cell 1 |  |  |  |  |  |
| 6.7.4.2.1 | NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy | NR Cell 1 |  |  |  |  |  |
| 6.7.4.2.2 | NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy | NR Cell 1 |  |  |  |  |  |

Table E.4-2 defines the cell configuration mapping for SA FR1 – E-UTRA Inter-RAT test cases (serving cell in NR) in chapter 6 of this test specification.

Table E.4-2: Cell configuration mapping for SA FR1 – E-UTRA Inter-RAT RRM testing

| TC | Description | 38.533 NR Cell1 | 38.533 LTE Cell2 | 38.533 LTE Cell3 | 38.533 LTE Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 6.1.2.1 | NR SA FR1 – E-UTRA cell re-selection to higher priority E-UTRA | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.1.2.2 | NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.1.2.3 | NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.1.2.4 | NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.1.2.5 | NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA for UE configured with highSpeedMeasFlag-r16 | NR Cell 1 | LTE Cell 11 |  |  |  |
| 6.3.1.4 | NR SA FR1 – E-UTRA handover with known target cell | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.3.1.5 | NR SA FR1 – E-UTRA handover with unknown target cell | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.3.2.3.2 | NR SA FR1 – E-UTRA RRC connection release with redirection | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.6.3.1 | NR SA FR1 – E-UTRAN event-triggered reporting in non-DRX | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.6.3.2 | NR SA FR1 – E-UTRAN event-triggered reporting in DRX | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.6.3.3 | NR SA FR1 – E-UTRAN event-triggered reporting in DRX for UE configured with highSpeedMeasFlag-r16 | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.7.5.1 | NR SA FR1 – E-UTRAN RSRP absolute measurement accuracy | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.7.6.1 | NR SA FR1 – E-UTRAN RSRQ absolute measurement accuracy | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.7.7.1 | NR SA FR1 – E-UTRAN RS-SINR absolute measurement accuracy | NR Cell 1 | LTE Cell 1 |  |  |  |

Table E.4-3: Cell configuration mapping for SA FR1 – UTRA FDD Inter-RAT RRM testing

| TC | Description | 38.533 NR Cell1 | 38.533 UTRA Cell2 | 38.533 UTRA Cell3 | 38.533 UTRA Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 6.3.1.6 | NR SA FR1 – UTRA cell re-selection to higher priority UTRA | NR Cell 1 | UTRA Cell 1 |  |  |  |
| 6.6.5.1 | NR SA FR1 – UTRAN event-triggered reporting in non-DRX | NR Cell 1 | UTRA Cell 1 |  |  |  |

# E.5	Cell configuration mapping for SA FR2 test cases in Chapter 7

Table E.5-1 defines the cell configuration mapping for SA FR2 test cases in chapter 7 of this test specification.

Table E.5-1: Cell configuration mapping for SA FR2 RRM testing

| TC | Description | 38.533 NR Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 7.1.1.1 | NR SA FR2 cell re-selection | NR Cell 1 | NR Cell 11 |  |  |  |
| 7.1.1.2 | NR SA FR2-FR2 cell re-selection | NR Cell 1 | NR Cell 23 |  |  |  |
| 7.1.1.3 | NR SA FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.1.1.4 | NR SA FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.1.1.5 | NR SA FR2-FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.1.1.6 | NR SA FR2-FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.3.1.4 | NR SA FR1-FR2 synchronous DAPS handover | NR Cell 1 | NR Cell 10 |  |  |  |
| 7.3.1.5 | NR SA FR1-FR2 asynchronous DAPS handover | NR Cell 1 | NR Cell 10 |  |  |  |
| 7.3.2.1.1 | NR SA FR2 RRC re-establishment | NR Cell 1 |  |  |  |  |
| 7.3.2.1.2 | NR SA FR2-FR2 re-establishment | NR Cell 1 |  |  |  |  |
| 7.3.2.1.3 | NR SA FR2 RRC re-establishment without serving cell timing | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.3.2.2.1 | NR SA FR2 contention based random access | NR Cell 1 |  |  |  |  |
| 7.3.2.2.2 | NR SA FR2 non-contention based random access | NR Cell 1 |  |  |  |  |
| 7.3.3.1 | NR SA FR2 conditional handover | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.3.3.2 | NR SA FR2-FR2 conditional handover | NR Cell 1 | NR Cell 6 |  |  |  |
| 7.5.1.9 | NR SA FR2 radio link monitoring UE scheduling restrictions | NR Cell 1 |  |  |  |  |
| 7.5.5.1 | NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX | NR Cell 1 |  |  |  |  |
| 7.5.5.2 | NR SA FR2 SSB-based beam failure detection and link recovery in DRX | NR Cell 1 |  |  |  |  |
| 7.5.5.3 | NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX | NR Cell 1 |  |  |  |  |
| 7.5.5.4 | NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX | NR Cell 1 |  |  |  |  |
| 7.5.6.1 | NR SA FR2 DCI-based DL active BWP switch in non-DRX | NR Cell 1 |  |  |  |  |
| 7.6.1.1 | NR SA FR2 event-triggered reporting without gap in non-DRX | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.6.1.2 | NR SA FR2 event-triggered reporting without gap in DRX | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.6.1.3 | NR SA FR2 event-triggered reporting with gap in non-DRX | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.6.1.4 | NR SA FR2 event-triggered reporting with gap in DRX | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.6.2.1 | NR SA FR2-FR2 event-triggered reporting in non-DRX | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.2.2 | NR SA FR2-FR2 event-triggered reporting in DRX | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.2.3 | NR SA FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.2.4 | NR SA FR2-FR2 event-triggered reporting in DRX with SSB time index detection | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.2.5 | NR SA FR1-FR2 event-triggered reporting in non-DRX | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.2.6 | NR SA FR1-FR2 event-triggered reporting in DRX | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.2.7 | NR SA FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.2.8 | NR SA FR1-FR2 event-triggered reporting in DRX with SSB time index detection | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.3.1 | NR SA FR2 SSB-based L1-RSRP measurement in non-DRX | NR Cell 1 |  |  |  |  |
| 7.6.3.2 | NR SA FR2 SSB-based L1-RSRP measurement in DRX | NR Cell 1 |  |  |  |  |
| 7.6.3.3 | NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX | NR Cell 1 |  |  |  |  |
| 7.6.3.4 | NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX | NR Cell 1 |  |  |  |  |
| 7.7.1.1 | NR SA FR2 SS-RSRP measurement accuracy | NR Cell 489 | NR Cell 1 |  |  |  |
| 7.7.1.2 | NR SA FR2-FR2 SS-RSRP measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.7.1.3.1 | NR SA FR1-FR2 SS-RSRP absolute measurement accuracy | NR Cell 1 | NR Cell 10 |  |  |  |
| 7.7.1.3.2 | Void |  |  |  |  |  |
| 7.7.2.1 | NR SA FR2 SS-RSRQ measurement accuracy | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.7.2.2 | NR SA FR2-FR2 SS-RSRQ measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.7.3.1 | NR SA FR2 SS-SINR measurement accuracy | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.7.3.2 | NR SA FR2-FR2 SS-SINR measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |

# E.6	Cell configuration mapping for E-UTRAN – SA test cases in Chapter 8

Table E.6-1 defines the cell configuration mapping for E-UTRAN - SA FR1 Inter-RAT test cases (serving cell in LTE) in chapter 8 of this test specification.

Table E.6-1: Cell configuration mapping for E-UTRA – SA FR1 Inter-RAT RRM testing

| TC | Description | 38.533 LTE Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 8.2.1.1 | E-UTRA – NR FR1 cell re-selection to higher priority NR target cell | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.2.1.2 | E-UTRA – NR FR1 Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16 | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.3.1.1 | E-UTRA – NR FR1 handover with unknown target cell | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.1.1 | E-UTRA – NR FR1 SFTD measurement delay in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.1.2 | E-UTRA – NR FR1 SFTD measurement delay in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.2.1 | E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.2.2 | E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.2.3 | E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.2.4 | E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.2.9 | E-UTRA – NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16 | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.5.1.1 | E-UTRA – NR FR1 SFTD measurement accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.5.2.1.1.1 | E-UTRA SS-RSRP absolute measurement accuracy of a NR FR1 neighbour cell | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.5.2.2.1 | Void | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.5.2.3.1 | E-UTRA SS-RSRP absolute measurement accuracy of a NR FR2 neighbour cell | LTE Cell 1 | NR Cell 1 |  |  |  |

Table E.6-2 defines the cell configuration mapping for E-UTRAN - SA FR2 Inter-RAT test cases (serving cell in LTE) in chapter 8 of this test specification.

Table E.6-2: Cell configuration mapping for E-UTRA – SA FR2 Inter-RAT RRM testing

| TC | Description | 38.533 LTE Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 8.4.2.5 | E-UTRA event-triggered reporting of a NR FR2 neighbour cell without SSB time index detection in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.2.6 | E-UTRA event-triggered reporting of a NR FR2 neighbour cell without SSB time index detection in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.2.7 | E-UTRA event-triggered reporting of a NR FR2 neighbour cell with SSB time index detection in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.2.8 | E-UTRA event-triggered reporting of a NR FR2 neighbour cell with SSB time index detection in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.5.2.1.2 | E-UTRA SS-RSRP absolute measurement accuracy of a NR FR2 neighbour cell | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.5.2.2.2 | SS-RSRQ | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.5.2.3.2 | E-UTRA SS-RSRQ absolute measurement accuracy of a NR FR1 neighbour cell | LTE Cell 1 | NR Cell 1 |  |  |  |

Annex F (normative):
Measurement uncertainties and test tolerances

The requirements of this clause apply to all tests in the present document.

# F.1	Measurement uncertainties and test tolerances for FR1 and FR2

## F.1.1	Acceptable uncertainty of test system (normative)

See TS 38.521-1 [17] annex F.1.

### F.1.1.1	Measurement of test environments

See TS 38.521-1 [17] Annex F1.1.

### F.1.1.2	Measurement of RRM requirements

This clause defines the maximum test system uncertainty for the RRM requirements. The maximum uncertainty values allowed for the typical RRM measurement uncertainty contributors is defined in Table F.1.1.2-1 and Table F.1.1.2-2. Unless explicitly stated for a particular test case, these maximum uncertainty values should be used as starting point to perform the test tolerance analysis in TR 38.903 [22] for each of the test cases. Specific test cases might require a tighter measurement uncertainty value for some of the contributors. Exceptions to the general values in Table F.1.1.2-1 and Table F.1.1.2-2 shall be handled case by case.

Table F.1.1.2-1: Maximum allowed measurement uncertainty values for the test system for FR1 (up to 6 GHz) and Cell BW ≤ 40 MHz

| MU contributor | Unit | Value | Comment |
| --- | --- | --- | --- |
| AWGN absolute power, Noc averaged over BWConfig | dB | ±1.5 |  |
| AWGN absolute power, Noc for Measurement PRB | dB | ±1.5 | Measurement PRB may be for example #RBJ to RBJ+19 for SSB-based, or selected RBs in the range #RB0 to RB105 for CSI-RS-based |
| Ratio of cell X signal / AWGN, Êsx / Noc averaged over BWConfig | dB | ±0.3 | Same as in LTE |
| Ratio of cell X signal / AWGN, Êsx / Noc for Measurement PRB | dB | ±0.3 | Measurement PRB may be for example #RBJ to RBJ+19 for SSB-based, or selected RBs in the range #RB0 to RB105 for CSI-RS-based |
| Fading profile uncertainty | dB | ±0.5 for 1 Tx ±0.7 for 2 Tx | Depends on fading profile, can be referenced from TS 38.101-4 [20] |
| AWGN and signal flatness* | dB | ±2.0 | Same as in LTE, can be referenced from TS 38.101-4 [20] |
| Uplink absolute power measurement | dB | ±1.5 | Same as in TS 38.521-1 [17] |
| Uplink relative power measurement | dB | ±0.7 | Same as in TS 38.521-1 [17] |
| Uplink signal transmit timing relative to downlink | Tc | ±112 |  |
| Relative transmit timing accuracy during UE timing adjustment | Tc | ±88 |  |
| Timing Advance Adjustment accuracy | Tc | ±88 |  |
| Note 1: The values in this table are specified per cell. Multi-cell test cases need to combined these values in the TT analysis in TR 38.903 Note 2:These values apply for cell BW ≤ 40 MHz. The maximum allowed measurement uncertainty for higher cell BW is FFS. Note 3: Void |  |  |  |

Table F.1.1.2-2: Maximum allowed measurement uncertainty values for the test system for FR2 (up to 40GHz) and Cell BW ≤ 400 MHz

| MU contributor | Unit | Value | Comment |
| --- | --- | --- | --- |
| AWGN absolute power, Noc averaged over BWConfig | dB | PC3: ±5.654,5 PC1: FFS |  |
| AWGN absolute power, Noc for Measurement PRB | dB | PC3: ±5.654,5 PC1: FFS | Measurement PRB may be for example #RBJ to RBJ+19 for SSB-based, or selected RBs in the range #RB0 to RB274 for CSI-RS-based |
| Ratio of cell X signal / AWGN, Êsx / Noc averaged over BWConfig | dB | ±0.3 |  |
| Ratio of cell X signal / AWGN, Êsx / Noc for Measurement PRB | dB | ±0.3 | Measurement PRB may be for example #RBJ to RBJ+19 for SSB-based, or selected RBs in the range #RB0 to RB274 for CSI-RS-based |
| ÊsxNote 3 absolute power, Noc averaged over BWConfig | dB | PC3: ±5.654,5 PC1: FFS |  |
| ÊsxNote 3 absolute power, Noc for Measurement PRB | dB | PC3: ±5.654,5 PC1: FFS | Measurement PRB may be for example #RBJ to RBJ+19 for SSB-based, or selected RBs in the range #RB0 to RB274 for CSI-RS-based |
| Relative power between any two Ês levels at the same frequency | dB | ±2.0 |  |
| Fading profile uncertainty | dB | ±0.5 for 1 Tx ±0.7 for 2 Tx |  |
| Uplink absolute power measurement | dB | TBD |  |
| Uplink relative power measurement | dB | TBD |  |
| Uplink signal transmit timing relative to downlink | Tc | ±48 |  |
| Relative transmit timing during UE timing adjustment | Tc | ±40 |  |
| Timing Advance Adjustment | Tc | ±40 |  |
| NOTE 1: The values in this table are specified per cell. Multi-cell test cases need to combine these values in the TT analysis in TR 38.903 NOTE 2: These values apply for cell BW ≤ 400 MHz. The maximum allowed measurement uncertainty for higher cell BW is FFS. NOTE 3: Applies for test cases that use Ês without AWGN NOTE 4: Total Expanded MU for IFF for Max device size ≤ 30cm in TR 38.903 [22] Table E.3.1.1-2 for PC3 UEs and in Table FFS for PC1 UEs. NOTE 5: If the TT analysis for a specific test case based on this MU value results in an unsolvable conflict, making the test case untestable, even after the alternative solutions listed in TR 38.903 [22] clause A.4 have been considered for the test case in TS 38.133 [6] Annex A, the TT analysis shall be repeated using a lower MU value, taking into account the values defined in clause E.3.1 TR 38.903 [22]. The test case will be applicable for the subset of the test systems meeting this reduced MU Threshold. |  |  |  |

The maximum test system uncertainty for the EN-DC FR1 test cases in chapter 4 is defined in Table F.1.1.2-3.

The maximum test system uncertainty for the NR SA FR1 test cases in chapter 6 is defined in Table F.1.1.2-4.

The maximum test system uncertainty for the EN-DC FR2 test cases in chapter 5 is defined in Table F.1.1.2-5.

The maximum test system uncertainty for the NR SA FR2 test cases in chapter 7 is defined in Table F.1.1.2-6.

Table F.1.1.2-3: Maximum test system uncertainty for RRM requirements for EN-DC FR1 test cases

| Subclause |  |  | Maximum Test System Uncertainty1 |  |  |  | Derivation of Test System Uncertainty |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4.3.2.2.1 Contention based random access test in FR1 for PSCell in EN-DC |  |  | Noc ±1.5 dB Ês2 / Noc ±0.3 dB Uplink absolute power measurement ±1.5 dB Uplink relative power measurement ±0.7 dB ±112Tc Uplink signal transmit timing relative to downlink |  |  |  | Ês2 / Noc is the ratio of cell 2 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |  |  |
| 4.3.2.2.2 Non-contention based random access test in FR1 for PSCell in EN-DC |  |  | Same as 4.3.2.2.1 |  |  |  | Same as 4.3.2.2.1 |  |  |
| 4.4.1.1 EN-DC FR1 UE transmit timing accuracy | Noc ±1.5 dB Ês2 / Noc ±0.3 dB ±112Tc Uplink signal transmit timing relative to downlink |  |  |  | Ês2 / Noc is the ratio of cell 2 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |  |  |  |
| 4.4.3.1 EN-DC FR1 timing advance adjustment accuracy |  |  | Noc ±1.5 dB Ês2 / Noc ±0.3 dB ±88Tc Timing Advance Adjustment accuracy |  |  |  | Ês2 / Noc is the ratio of cell 2 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |  |  |
| 4.5.1.1 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode |  |  | Average Noc2 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Fading profile ±0.7 dB |  |  |  | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the SNR Meas PRB are the measurement PRB for SSB-based RLM #RBJ to RBJ+19 |  |  |
| 4.5.1.2 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode |  |  | Same as 4.5.1.1 |  |  |  | Same as 4.5.1.1 |  |  |
| 4.5.1.3 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode |  |  | Same as 4.5.1.1 |  |  |  | Same as 4.5.1.1 |  |  |
| 4.5.1.4 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode |  |  | Same as 4.5.1.1 |  |  |  | Same as 4.5.1.1 |  |  |
| 4.5.1.5 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode |  |  | Same as 4.5.1.1 |  |  |  | Same as 4.5.1.1 |  |  |
| 4.5.1.6 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode |  |  | Same as 4.5.1.1 |  |  |  | Same as 4.5.1.1 |  |  |
| 4.5.1.7 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode |  |  | Same as 4.5.1.1 |  |  |  | Same as 4.5.1.1 |  |  |
| 4.5.1.8 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode |  |  | Same as 4.5.1.1 |  |  |  | Same as 4.5.1.1 |  |  |
| 4.5.2.1 EN-DC FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC |  |  | ± 0.6dB (AWGN conditions) |  |  |  | Overall system uncertainty for AWGN condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |  |  |
| 4.5.2.2 EN-DC FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC |  |  | Same as 4.5.2.1 |  |  |  | Same as 4.5.2.1 |  |  |
| 4.5.2.3 EN-DC FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC |  |  | Noc2 ±1.5 dB Noc3 ±1.5 dB Ês2 / Noc2 ±0.3 dB Ês3 / Noc3 ±0.3 dB |  |  |  | Note: Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc3 is the ratio of cell 3 signal / AWGN |  |  |
| 4.5.2.4 EN-DC FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC |  |  | Same as 4.5.2.3 |  |  |  | Same as 4.5.2.3 |  |  |
| 4.5.2.5 EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC |  |  | Noc1 ±1.5 dB Noc2 ±1.5 dB Noc3 ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Ês3 / Noc ±0.3 dB |  |  |  | Note: Noc1 is the AWGN on E-UTRAN cell 1 frequency Ês1 / Noc1 is the ratio of E-UTRAN cell 1 signal / AWGN Noc2 is the AWGN on NR cell 2 frequency Ês2 / Noc2 is the ratio of NR cell 2 signal / AWGN Noc3 is the AWGN on E-UTRAN cell 3 frequency Ês3 / Noc3 is the ratio of E-UTRAN cell 3 signal / AWGN |  |  |
| 4.5.2.6 EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC |  |  | Same as 4.5.2.5 |  |  |  | Same as 4.5.2.5 |  |  |
| 4.5.3.1 EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle |  |  | Average Noc2 ±1.5 dB Average Noc3 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Average Ês3 / Noc3 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Noc3 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Meas PRB Ês3 / Noc3 ±0.3 dB |  |  |  | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on cell 3 frequency Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |
| 4.5.3.2 EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 320ms SCell measurement cycle |  |  | Same as 4.5.3.1 |  |  |  | Same as 4.5.3.1 |  |  |
| 4.5.3.3 EN-DC FR1 SCell activation and deactivation of unknown SCell in non-DRX |  |  | Same as 4.5.3.1 |  |  |  | Same as 4.5.3.1 |  |  |
| 4.5.4.1 EN-DC FR1 UE UL carrier RRC reconfiguration delay |  |  |  | Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB Noc3 ±1.5 dB Ês3 / Noc3 ±0.3 dB |  |  |  | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on cell 3 frequency Ês3 / Noc3 is the ratio of cell 3 signal / AWGN |  |  |
| 4.5.5.1 EN-DC FR1 SSB-based beam failure detection and link recovery in non-DRX |  |  |  | Average Noc2 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Fading profile ±0.7 dB |  |  |  | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the SNR for the SSB Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |
| 4.5.5.2 EN-DC FR1 SSB-based beam failure detection and link recovery in DRX |  |  |  | Same as 4.5.5.1 |  |  |  | Same as 4.5.5.1 |  |  |
| 4.5.5.3 EN-DC FR1 CSI-RS-based beam failure detection and link recovery in non-DRX |  |  |  | Average Noc2 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Fading profile ±0.7 dB |  |  |  | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the SNR for the CSI-RS Meas PRB is the measurement PRB for CSI-RSRP #RB0 to RB274 |  |  |
| 4.5.5.4 EN-DC FR1 CSI-RS-based beam failure detection and link recovery in DRX |  |  |  | Same as 4.5.5.3 |  |  |  | Same as 4.5.5.3 |  |  |
| 4.5.6.1.1 EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC |  |  |  | ±0.6 dB |  |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty^2 + (0.25 x AWGN flatness and signal flatness)^2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |  |  |
| 4.5.6.1.2 EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC |  |  |  | Same as 4.5.6.1.1 |  |  |  | Same as 4.5.6.1.1 |  |  |
| 4.5.6.2.1 EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC |  |  |  | ±0.6 dB |  |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty^2 + (0.25 x AWGN flatness and signal flatness)^2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |  |  |
| 4.5.7.1 EN-DC FR1 addition and release delay of known PSCell |  | Average Noc1 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB |  |  |  | Noc1 is the AWGN on NR freq1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |  |
| 4.5.8.1 EN-DC FR1 interruptions at switching between two uplink carriers |  | Average Noc ±1.5 dB Average Ês / Noc ±0.3 dB |  |  |  | Noc is the AWGN on cell 2 frequency Ês / Noc is the ratio of cell 2 signal / AWGN |  |  |  |
| 4.6.1.1 EN-DC FR1 event-triggered reporting without gap in non-DRX |  |  | Noc ±1.5 dB Ês2 / Noc ±0.3 dB Ês3 / Noc ±0.3 dB |  |  |  | Note: Ês2 / Noc is the ratio of cell 2 signal / AWGN Ês3 / Noc is the ratio of cell 3 signal / AWGN |  |  |
| 4.6.1.2 EN-DC FR1 event-triggered reporting without gap in DRX |  |  | Same as 4.6.1.1 |  |  |  | Same as 4.6.1.1 |  |  |
| 4.6.1.3 EN-DC FR1 event-triggered reporting with gap in non-DRX |  |  | Same as 4.6.1.1 |  |  |  | Same as 4.6.1.1 |  |  |
| 4.6.1.4 EN-DC FR1 event-triggered reporting with gap in DRX |  |  | Same as 4.6.1.1 |  |  |  | Same as 4.6.1.1 |  |  |
| 4.6.1.5 EN-DC FR1 event-triggered reporting without gap in non-DRX with SSB time index detection | Same as 4.6.1.1 |  |  |  | Same as 4.6.1.1 |  |  |  |
| 4.6.1.6 EN-DC FR1 event-triggered reporting with gap in non-DRX with SSB time index detection | Same as 4.6.1.1 |  |  |  | Same as 4.6.1.1 |  |  |  |
| 4.6.1.7 EN-DC FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 |  |  | Same as 4.6.1.1 |  |  |  | Same as 4.6.1.1 |  |  |
| 4.6.2.1 EN-DC FR1-FR1 event-triggered reporting in non-DRX |  |  | Average Noc2 ±1.5 dB Average Noc3 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Average Ês3 / Noc3 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Noc3 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Meas PRB Ês3 / Noc3 ±0.3 dB |  |  |  | Noc2 is the AWGN on NR freq2 Noc3 is the AWGN on NR freq3 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |
| 4.6.2.2 EN-DC FR1-FR1 event-triggered reporting in DRX |  |  | Same as 4.6.2.1 |  |  |  | Same as 4.6.2.1 |  |  |
| 4.6.2.5 EN-DC FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection |  |  | Same as 4.6.2.1 |  |  |  | Same as 4.6.2.1 |  |  |
| 4.6.2.6 EN-DC FR1-FR1 event-triggered reporting in DRX with SSB time index detection |  |  | Same as 4.6.2.1 |  |  |  | Same as 4.6.2.1 |  |  |
| 4.6.4.1 EN-DC FR1 SSB-based L1-RSRP measurement in non-DRX |  |  | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB |  |  |  | Noc is the AWGN on NR freq1 Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |
| 4.6.4.2 EN-DC FR1 SSB-based L1-RSRP measurement in DRX |  |  | Same as 4.6.4.1 |  |  |  | Same as 4.6.4.1 |  |  |
| 4.6.4.3 EN-DC FR1 CSI-RS-based L1-RSRP measurement in non-DRX |  |  | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB |  |  |  | Noc is the AWGN on NR freq1 Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-RSRP #RB0 to RB274 |  |  |
| 4.6.4.4 EN-DC FR1 CSI-RS-based L1-RSRP measurement in DRX |  |  | Same as 4.6.4.3 |  |  |  | Same as 4.6.4.3 |  |  |
| 4.6.4.5 EN-DC FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16 |  |  | Same as 4.6.4.1 |  |  |  | Same as 4.6.4.1 |  |  |
| 4.7.1.1.1 EN-DC FR1 SS-RSRP absolute measurement accuracy |  |  | Average Noc ±1.5 dB Average Ês2 / Noc ±0.3 dB Average Ês3 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês2 / Noc ±0.3 dB Meas PRB Ês3 / Noc ±0.3 dB |  |  |  | Noc is the AWGN on NR freq1 Ês2 / Noc is the cell 2 SNR Ês3 / Noc is the cell 3 SNR Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |
| 4.7.1.1.2 EN-DC FR1 SS-RSRP relative measurement accuracy |  |  | Same as 4.7.1.1.1 |  |  |  | Same as 4.7.1.1.1 |  |  |
| 4.7.1.2.1 EN-DC FR1-FR1 SS-RSRP absolute measurement accuracy |  |  | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês2 / Noc1 ±0.3 dB Average Ês3 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc1 ±0.3 dB Meas PRB Ês3 / Noc2 ±0.3 dB |  |  |  | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês2 / Noc1 is the cell 2 SNR Ês3 / Noc2 is the cell 3 SNR Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |
| 4.7.1.2.2 EN-DC FR1-FR1 SS-RSRP relative measurement accuracy |  |  | Same as 4.7.1.2.1 |  |  |  | Same as 4.7.1.2.1 |  |  |
| 4.7.2.1 EN-DC FR1 SS-RSRQ measurement accuracy |  |  | Same as 4.7.1.1.1 |  |  |  | Same as 4.7.1.1.1 |  |  |
| 4.7.2.2.1 EN-DC FR1-FR1 SS-RSRQ absolute measurement accuracy |  |  | Same as 4.7.1.2.1 |  |  |  | Same as 4.7.1.2.1 |  |  |
| 4.7.2.2.2 EN-DC FR1-FR1 SS-RSRQ relative measurement accuracy |  |  | Same as 4.7.1.2.1 |  |  |  | Same as 4.7.1.2.1 |  |  |
| 4.7.3.1 EN-DC FR1 SS-SINR measurement accuracy |  |  | Same as 4.7.1.1.1 |  |  |  | Same as 4.7.1.1.1 |  |  |
| 4.7.3.2.1 EN-DC FR1-FR1 SS-SINR absolute measurement accuracy |  |  | Same as 4.7.1.2.1 |  |  |  | Same as 4.7.1.2.1 |  |  |
| 4.7.3.2.2 EN-DC FR1-FR1 SS-SINR relative measurement accuracy |  |  | Same as 4.7.1.2.1 |  |  |  | Same as 4.7.1.2.1 |  |  |
| 4.7.4.1.1 EN-DC FR1 SSB based L1-RSRP absolute measurement accuracy |  |  | Average Noc ±1.5 dB Average Ês2 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.8 dB |  |  |  | Noc is the AWGN on NR freq1 Ês2 / Noc is the cell 2 SNR Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |
| 4.7.4.1.2 EN-DC FR1 SSB based L1-RSRP relative measurement accuracy |  |  | Same as 4.7.4.1.1 |  |  |  | Same as 4.7.4.1.1 |  |  |
| 4.7.4.2.1 EN-DC FR1 CSI-RS based L1-RSRP absolute measurement accuracy |  |  | Same as 4.7.4.1.1 |  |  |  | Same as 4.7.4.1.1 |  |  |
| 4.7.4.2.2 EN-DC FR1 CSI-RS based L1-RSRP relative measurement accuracy |  |  | Same as 4.7.4.1.1 |  |  |  | Same as 4.7.4.1.1 |  |  |
| 4.7.5.1 EN-DC FR1 SFTD measurement accuracy |  |  | Average Noc1 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Average Noc2 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.8 dB |  |  |  | Noc1 is the AWGN on NR freq1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 Noc2 is the AWGN on E-UTRA freq2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RB22 to RB27 |  |  |

Table F.1.1.2-4 Maximum test system uncertainty for RRM requirements for SA FR1 test cases

| Subclause | Maximum Test System Uncertainty1 |  | Derivation of Test System Uncertainty |
| --- | --- | --- | --- |
| 6.1.1.1 NR SA FR1 cell re-selection | Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB |  | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 6.1.1.2 NR SA FR1-FR1 cell re-selection | Noc1 ±1.5 dB Ês1 / Noc1 ±0.3 dB Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB |  | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 6.1.1.3 NR SA FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | Same as 6.1.1.1 |  | Same as 6.1.1.1 |
| 6.1.1.4 NR SA FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | Same as 6.1.1.1 |  | Same as 6.1.1.1 |
| 6.1.1.5 NR SA FR1-FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | Same as 6.1.1.2 |  | Same as 6.1.1.2 |
| 6.1.1.6 NR SA FR1-FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | Same as 6.1.1.2 |  | Same as 6.1.1.2 |
| 6.1.2.1 NR SA FR1 – E-UTRA cell re-selection to higher priority E-UTRA | Noc1 ±1.5 dB Ês1 / Noc1 ±0.3 dB Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB |  | Note: Noc1 is the AWGN on cell 1 (NR) frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 (E-UTRAN) frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 6.1.2.2 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA | Same as 6.1.2.1 |  | Same as 6.1.2.1 |
| 6.1.2.3 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion | Same as 6.1.2.1 |  | Same as 6.1.2.1 |
| 6.1.2.4 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion | Same as 6.1.2.1 |  | Same as 6.1.2.1 |
| 6.1.2.5 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA for UE configured with highSpeedMeasFlag-r16 | Same as 6.1.2.1 | Same as 6.1.2.1 |  |
| 6.3.1.1 NR SA FR1 handover with known target cell | Average Noc ±1.5 dB Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 6.3.1.2 NR SA FR1 handover with unknown target cell | Same as 6.3.1.1 | Same as 6.3.1.1 |  |
| 6.3.1.3 NR SA FR1-FR1 Handover with unknown Target Cell | Noc1 ±1.5 dB Ês1 / Noc1 ±0.3 dB Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |  |
| 6.3.1.4 NR SA FR1 – E-UTRA handover with known target cell | Same as 6.1.2.1 |  | Same as 6.1.2.1 |
| 6.3.1.5 NR SA FR1 – E-UTRA handover with unknown target cell | Same as 6.1.2.1 |  | Same as 6.1.2.1 |
| 6.3.1.6 NR SA FR1 – UTRAN FDD handover with known target cell | Noc ±1.5 dB Ês / Noc ±0.3 dB Ioc ±0.7 dB Îor / Ioc ± 0.3dB CPICH Ec / Ior ± 0.1dB SCH Ec / Ior ± 0.1dB |  | Note: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 frequency Îor / Ioc is the ratio of cell 2 signal / AWGN CPICH Ec / Ior is the ratio of cell 2 CPICH code level / Ior SCH Ec / Ior is the ratio of cell 2 SCH code level / Ior |
| 6.3.1.7 NR SA FR1 synchronous DAPS handover | Same as 6.3.1.1 |  | Same as 6.3.1.1 |
| 6.3.1.8 NR SA FR1 asynchronous DAPS handover | Same as 6.3.1.1 |  | Same as 6.3.1.1 |
| 6.3.1.9 NR SA FR1 Intra-band inter-frequency synchronous DAPS handover | Same as 6.3.1.3 |  | Same as 6.3.1.3 |
| 6.3.1.10 NR SA FR1 Intra-band inter-frequency asynchronous DAPS handover | Same as 6.3.1.3 |  | Same as 6.3.1.3 |
| 6.3.2.1.1 NR SA FR1 RRC re-establishment | Same as 6.1.1.1 |  | Same as 6.1.1.1 |
| 6.3.2.1.2 NR SA FR1 - FR1 RRC re-establishment | Same as 6.1.1.2 |  | Same as 6.1.1.2 |
| 6.3.2.1.3 NR SA FR1 RRC re-establishment without serving cell timing | Average Noc ±1.5 dB Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB |  | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.3.2.2.1 | Noc ±1.5 dB Ês / Noc ±0.3 dB Uplink absolute power measurement ±1.5 dB Uplink relative power measurement ±0.7 dB ±112Tc Uplink signal transmit timing relative to downlink |  | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 6.3.2.2.2 | Same as 6.3.2.2.1 |  | Same as 6.3.2.2.1 |
| 6.3.2.3.1 NR SA FR1 RRC connection release with redirection | Same as 6.1.1.2 |  | Same as 6.1.1.2 |
| 6.3.2.3.2 NR SA FR1 – E-UTRA RRC connection release with redirection | Same as 6.1.2.1 |  | Same as 6.1.2.1 |
| 6.3.3.1 NR SA FR1 conditional handover | Same as 6.3.1.1 |  | Same as 6.3.1.1 |
| 6.3.3.2 NR SA FR1-FR1 conditional handover | Same as 6.3.1.3 |  | Same as 6.3.1.3 |
| 6.4.1.1 EN-DC FR1 UE transmit timing accuracy | Noc ±1.5 dB Ês1 / Noc ±0.3 dB ±112Tc Uplink signal transmit timing relative to downlink |  | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 6.4.3.1 NR SA FR1 timing advance adjustment accuracy | Noc ±1.5 dB Ês1 / Noc ±0.3 dB ±88Tc Timing Advance Adjustment accuracy |  | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 6.5.1.1 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode | Average Noc ±1.5 dB Average Ês / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês / Noc ±0.3 dB Fading profile ±0.7 dB |  | Noc is the AWGN on cell 1 frequency Ês / Noc is the SNR Meas PRB are the measurement PRB for SSB-based RLM #RBJ to RBJ+19 |
| 6.5.1.2 NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  |
| 6.5.1.3 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  |
| 6.5.1.4 NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  |
| 6.5.1.5 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  |
| 6.5.1.6 NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  |
| 6.5.1.7 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  |
| 6.5.1.8 NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  |
| 6.5.2.1 NR SA FR1 interruptions during measurements on deactivated NR SCC | Noc1 ±1.5 dB Noc2 ±1.5 dB Ês1 / Noc1 ±0.3 dB Ês2 / Noc2 ±0.3 dB |  | Note: Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |  |
| 6.5.3.1 NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB |  | Note: Noc1 is the AWGN on cell 1 frequency Noc2 is the AWGN on cell 2 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 6.5.3.2 NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 320ms SCell measurement cycle | Same as 6.5.3.1 |  | Same as 6.5.3.1 |  |
| 6.5.3.3 NR SA FR1 SCell activation and deactivation of unknown SCell in non-DRX | Same as 6.5.3.1 |  | Same as 6.5.3.1 |  |
| 6.5.5.1 NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX | Average Noc1 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Fading profile ±0.7 dB |  | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the SNR for the SSB Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 6.5.5.2 NR SA FR1 SSB-based beam failure detection and link recovery in DRX | Same as 6.5.5.1 |  | Same as 6.5.5.1 |  |
| 6.5.5.3 NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX | Average Noc1 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Fading profile ±0.7 dB |  | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the SNR for the CSI-RS Meas PRB is the measurement PRB for CSI-RSRP #RB0 to RB274 |  |
| 6.5.5.4 NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX | Same as 6.5.5.3 |  | Same as 6.5.5.3 |  |
| 6.5.6.1.1 NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX | ±0.6 dB |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty^2 + (0.25 x AWGN flatness and signal flatness)^2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |  |
| 6.5.6.1.2 NR SA FR1 DCI-based DL active BWP switch in non-DRX | Same as 6.5.6.1.1 |  | Same as 6.5.6.1.1 |  |
| 6.5.6.2.1 NR SA FR1 RRC-based DL active BWP switch in non-DRX | ±0.6 dB |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty^2 + (0.25 x AWGN flatness and signal flatness)^2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |  |
| 6.6.1.1 SA event triggered reporting tests without gap under non-DRX | Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB |  | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |  |
| 6.6.1.2 SA event triggered reporting tests without gap under DRX | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  |
| 6.6.1.3 SA event triggered reporting tests with per-UE gaps under non-DRX | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  |
| 6.6.1.4 SA event triggered reporting tests with per-UE gaps under DRX | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  |
| 6.6.1.5 SA event triggered reporting tests without gap under non-DRX with SSB index reading | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  |
| 6.6.1.6 SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  |
| 6.6.1.7 NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  |
| 6.6.2.1 NR SA FR1-FR1 event-triggered reporting in non-DRX | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB |  | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 6.6.2.2 NR SA FR1-FR1 event-triggered reporting in DRX | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  |
| 6.6.2.5 NR SA FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  |
| 6.6.2.6 NR SA FR1-FR1 event-triggered reporting in DRX with SSB time index detection | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  |
| 6.6.3.1 NR SA FR1 – E-UTRAN event-triggered reporting in non-DRX | Freq 1 Noc ±1.5 dB Freq 2 Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Fading ±0.5 dB |  | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |  |
| 6.6.3.2 NR SA FR1 – E-UTRAN event-triggered reporting in DRX | Same as 6.6.3.1 |  | Same as 6.6.3.1 |  |
| 6.6.3.3 NR SA FR1 – E-UTRAN event-triggered reporting in DRX for UE configured with highSpeedMeasFlag-r16 | Freq 1 Noc ±1.5 dB Freq 2 Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB |  | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |  |
| 6.6.4.1 NR SA FR1 SSB-based L1-RSRP measurement in non-DRX | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB |  | Noc is the AWGN on NR freq1 Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 6.6.4.2 NR SA FR1 SSB-based L1-RSRP measurement in DRX | Same as 6.6.4.1 |  | Same as 6.6.4.1 |  |
| 6.6.4.3 NR SA FR1 CSI-RS-based L1-RSRP measurement in non-DRX | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB |  | Noc is the AWGN on NR freq1 Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-RSRP #RB0 to RB274 |  |
| 6.6.4.4 NR SA FR1 CSI-RS-based L1-RSRP measurement in DRX | Same as 6.6.4.3 |  | Same as 6.6.4.3 |  |
| 6.6.4.5 NR SA FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16 | Same as 6.6.4.1 |  | Same as 6.6.4.1 |  |
| 6.6.5.1 NR SA FR1 – UTRAN event-triggered reporting in non-DRX | Noc ±1.5 dB Ês / Noc ±0.3 dB Ioc ±0.7 dB Îor / Ioc ± 0.3dB CPICH Ec / Ior ± 0.1dB SCH Ec / Ior ± 0.1dB |  | Note: Noc is the AWGN on cell 1 frequency Ês / Noc is the ratio of cell 1 signal / AWGN Ioc is the AWGN on cell 2 frequency Îor / Ioc is the ratio of cell 2 signal / AWGN CPICH Ec / Ior is the ratio of cell 2 CPICH code level / Ior SCH Ec / Ior is the ratio of cell 2 SCH code level / Ior |  |
| 6.7.1.1.1 NR SA FR1 SS-RSRP absolute measurement accuracy | Average Noc ±1.5 dB Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB meas PRB Ês1 / Noc ±0.3 dB meas PRB Ês2 / Noc ±0.3 dB |  | Noc is the AWGN on NR freq1 Ês1 / Noc is the cell 1 SNR Ês2 / Noc is the cell 2 SNR meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 6.7.1.1.2 NR SA FR1 SS-RSRP relative measurement accuracy | Same as 6.7.1.1.1 |  | Same as 6.7.1.1.1 |  |
| 6.7.1.2.1 NR SA FR1-FR1 SS-RSRP absolute measurement accuracy | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB meas PRB Ês1 / Noc1 ±0.3 dB meas PRB Ês2 / Noc2 ±0.3 dB |  | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês1 / Noc1 is the cell 2 SNR Ês2 / Noc2 is the cell 3 SNR meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.7.1.2.2 NR SA FR1-FR1 SS-RSRP relative measurement accuracy | Same as 6.7.1.2.1 |  | Same as 6.7.1.2.1 |
| 6.7.2.1 NR SA FR1 SS-RSRQ measurement accuracy | Same as 6.7.1.1.1 |  | Same as 6.7.1.1.1 |
| 6.7.2.2.1 NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy | Same as 6.7.1.2.1 |  | Same as 6.7.1.2.1 |
| 6.7.2.2.2 NR SA FR1-FR1 SS-RSRQ relative measurement accuracy | Same as 6.7.1.2.1 |  | Same as 6.7.1.2.1 |
| 6.7.3.1 NR SA FR1 SS-SINR measurement accuracy | Same as 6.7.1.1.1 |  | Same as 6.7.1.1.1 |
| 6.7.3.2.1 NR SA FR1-FR1 SS-SINR absolute measurement accuracy | Same as 6.7.1.2.1 |  | Same as 6.7.1.2.1 |
| 6.7.3.2.2 NR SA FR1-FR1 SS-SINR relative measurement accuracy | Same as 6.7.1.2.1 |  | Same as 6.7.1.2.1 |
| 6.7.4.1.1 NR SA FR1 SSB based L1-RSRP absolute measurement accuracy | Average Noc ±1.5 dB Average Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.8 dB |  | Noc is the AWGN on NR freq1 Ês2 / Noc is the cell 1 SNR Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 6.7.4.1.2 NR SA FR1 SSB based L1-RSRP relative measurement accuracy | Same as 6.7.4.1.1 |  | Same as 6.7.4.1.1 |
| 6.7.4.2.1 NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy | Same as 6.7.4.1.1 |  | Same as 6.7.4.1.1 |
| 6.7.4.2.2 NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy | Same as 6.7.4.1.1 |  | Same as 6.7.4.1.1 |
| 6.7.5.1 NR SA FR1 – E-UTRAN RSRP absolute measurement accuracy | Average Noc ±1.5 dB Average Ês / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês / Noc ±0.8 dB |  | Noc is the AWGN on E-UTRA freq1 Ês / Noc is the ratio of cell 1 signal / AWGN Meas PRB are the measurement PRB for RSRP #RB22 to RB27 |
| 6.7.6.1 NR SA FR1 – E-UTRAN RSRQ absolute measurement accuracy | Same as 6.7.5.1 |  | Same as 6.7.5.1 |
| 6.7.7.1 NR SA FR1 – E-UTRAN RS-SINR absolute measurement accuracy | Same as 6.7.5.1 |  | Same as 6.7.5.1 |

Table F.1.1.2-5: Maximum test system uncertainty for RRM requirements for EN-DC FR2 test cases

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 5.3.2.2.1 EN-DC FR2 contention based random access | Average Ês ±6.0 dB Meas PRB Ês ±6.0 dB Uplink absolute power measurement ±6.0 dB Uplink relative power measurement ±FFS dB ±[48]Tc Uplink signal transmit timing relative to downlink | Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 5.3.2.2.2 EN-DC FR2 non-contention based random access | Same as 5.3.2.2.1 | Same as 5.3.2.2.1 |
| 5.4.1.1 EN-DC FR2 UE transmit timing accuracy | Noc ±5.65 dB, f<=40.8 GHz Ês2 / Noc ±0.3 dB ±48Tc Uplink signal transmit timing relative to downlink ±40Tc Relative during UE timing adjustment | Ês2 / Noc is the ratio of cell 2 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 5.4.3.1 EN-DC FR2 UE timing advance adjustment accuracy | Noc ±5.65 dB, f<=40.8 GHz Ês1 / Noc ±0.3 dB ±40Tc timing advance adjustment | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 5.5.2.1 EN-DC FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC | Noc ±5.65 dB, f<=40.8 GHz Ês2 / Noc ±0.3 dB | Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 5.5.2.2 EN-DC FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC | Same as 5.5.2.1 | Same as 5.5.2.1 |
| 5.6.1.2 EN-DC FR2 event-triggered reporting without gap in DRX | Noc ±5.65 dB Ês2 / Noc ±0.3 dB Ês3 / Noc ±0.3 dB | Ês2 / Noc is the ratio of cell 2 signal / AWGN Ês3 / Noc is the ratio of cell 3 signal / AWGN |
| 5.6.1.4 EN-DC FR2 event-triggered reporting with gap in DRX | Same as 5.6.1.2 | Same as 5.6.1.2 |
| 5.6.2.1 EN-DC FR2-FR2 event-triggered reporting in non-DRX | Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz Average Ês2 ±5.65 dB Meas PRB Ês2 ±5.65 dB | Ês1 is NR cell 2 signal Ês2 is NR cell 3 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.6.2.2 EN-DC FR2-FR2 event-triggered reporting in DRX | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB Average Ês2 / Noc ±0.3 dB Ês2 / Noc over meas PRBs ±0.3 dB | Ês1 is NR cell 2 signal Ês2 is NR cell 3 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.6.2.3 EN-DC FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | Same as 5.6.2.1 | Same as 5.6.2.1 |
| 5.6.2.4 EN-DC FR2-FR2 event-triggered reporting in DRX with SSB time index detection | Same as 5.6.2.2 | Same as 5.6.2.2 |
| 5.6.2.5 EN-DC FR1-FR2 event-triggered reporting in non-DRX | Average Ês2 ±5.65, f <= 40.8 GHz | Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.6.2.6 EN-DC FR1-FR2 event-triggered reporting in DRX | Noc ±5.65 dB, f <= 40.8 GHz Ês2 / Noc ±0.3 dB | Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 5.6.2.7 EN-DC FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection | Same as 5.6.2.5 | Same as 5.6.2.5 |
| 5.6.2.8 EN-DC FR1-FR2 event-triggered reporting in DRX with SSB time index detection | Same as 5.6.2.6 | Same as 5.6.2.6 |
| 5.6.3.1 EN-DC FR2 SSB-based L1-RSRP measurement in non-DRX | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês0 / Noc ±0.3 dB Ês0 / Noc over meas PRBs ±0.3 dB Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB | Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.6.3.2 EN-DC FR2 SSB-based L1-RSRP measurement in DRX | Same as 5.6.3.1 | Same as 5.6.3.1 |
| 5.6.3.3 EN-DC FR2 CSI-RS-based L1-RSRP measurement in non-DRX | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês0 / Noc ±0.3 dB Ês0 / Noc over meas PRBs ±0.3 dB Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB | Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-RS-RSRP |
| 5.6.3.4 EN-DC FR2 CSI-RS-based L1-RSRP measurement in DRX | Same as 5.6.3.3 | Same as 5.6.3.3 |
| 5.7.1.1 EN-DC FR2 SS-RSRP measurement accuracy | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB Test 2 Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz Average Ês2 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês2 ±5.65 dB, f <= 40.8 GHz | Test 1 Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 2 signal / AWGN Ês2 / Noc is the ratio of cell 3 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Test 2 Ês1 is NR cell 2 signal Ês2 is NR cell 3 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.7.1.2 EN-DC FR2-FR2 SS-RSRP measurement accuracy | Noc1 ±5.65 dB, f <= 40.8 GHz Noc1 over measPRBs ±5.65 dB, f <= 40.8 GHz Noc2 ±5.65 dB, f <= 40.8 GHz Noc2 over measPRBs ±5.65 dB, f <= 40.8 GHz Ês1 / Noc1 ±0.3 dB Ês2 / Noc2 ±0.3 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Noc1 is the AWGN on the NR freq 1 Noc2 is the AWGN on the NR freq 2 Ês1 / Noc1 is the ratio of cell 2 signal / AWGN Ês2 / Noc2 is the ratio of cell 3 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.7.2.1 EN-DC FR2 SS-RSRQ measurement accuracy | Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB, f <= 40.8 GHz Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB | Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 2 signal / AWGN Ês2 / Noc is the ratio of cell 3 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.7.2.2 EN-DC FR2-FR2 SS-RSRQ measurement accuracy | Same as 5.7.1.2 | Same as 5.7.1.2 |
| 5.7.3.1 EN-DC FR2 SS-SINR measurement accuracy | Same as 5.7.2.1 | Same as 5.7.2.1 |
| 5.7.3.2 EN-DC FR2-FR2 SS-SINR measurement accuracy | Same as 5.7.1.2 | Same as 5.7.1.2 |

Table F.1.1.2-6: Maximum test system uncertainty for RRM requirements for SA FR2 test cases

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 7.3.2.2.1 NR SA FR2 contention based random access | Average Ês ±6.0 dB Meas PRB Ês ±6.0 dB Uplink absolute power measurement ±6.0 dB Uplink relative power measurement ±FFS dB ±[48]Tc Uplink signal transmit timing relative to downlink | Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 7.3.2.2.2 NR SA FR2 non-contention based random access | Same as 7.3.2.2.1 | Same as 7.3.2.2.1 |
| 7.4.1.1 SA FR2 UE transmit timing accuracy | Noc ±5.65 dB Ês1 / Noc ±0.3 dB ±48Tc Uplink signal transmit timing relative to downlink ±40Tc Relative during UE timing adjustment | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 7.4.3.1 NR SA FR2 UE timing advance adjustment accuracy | Noc ±5.65 dB Ês1 / Noc ±0.3 dB ±40Tc timing advance adjustment | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 7.6.2.1 NR SA FR2-FR2 event-triggered reporting in non-DRX | Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz Average Ês2 ±5.65 dB Meas PRB Ês2 ±5.65 dB | Ês1 is NR cell 1 signal Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 7.6.2.2 NR SA FR2-FR2 event-triggered reporting in DRX | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB Average Ês2 / Noc ±0.3 dB Ês2 / Noc over meas PRBs ±0.3 dB | Ês1 is NR cell 1 signal Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 7.6.2.3 NR SA FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | Same as 7.6.2.1 | Same as 7.6.2.1 |
| 7.6.2.4 NR SA FR2-FR2 event-triggered reporting in DRX with SSB time index detection | Same as 7.6.2.2 | Same as 7.6.2.2 |
| 7.6.2.5 NR SA FR1-FR2 event-triggered reporting in non-DRX | Average Ês2 ±5.65 dB, f <= 40.8 GHz | Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 7.6.2.6 NR SA FR1-FR2 event-triggered reporting in DRX | Noc ±5.65 dB, f <= 40.8 GHz Ês2 / Noc ±0.3 dB | Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 7.6.2.7 NR SA FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection | Same as 7.6.2.5 | Same as 7.6.2.5 |
| 7.6.2.8 NR SA FR1-FR2 event-triggered reporting in DRX with SSB time index detection | Same as 7.6.2.6 | Same as 7.6.2.6 |
| 7.6.3.1 NR SA FR2 SSB-based L1-RSRP measurement in non-DRX | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês0 / Noc ±0.3 dB Ês0 / Noc over meas PRBs ±0.3 dB Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB | Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 7.6.3.2 NR SA FR2 SSB-based L1-RSRP measurement in DRX | Same as 7.6.3.1 | Same as 7.6.3.1 |
| 7.6.3.3 NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês0 / Noc ±0.3 dB Ês0 / Noc over meas PRBs ±0.3 dB Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB | Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-RS-RSRP |
| 7.6.3.4 NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX | Same as 7.6.3.3 | Same as 7.6.3.3 |
| 7.7.1.1 NR SA FR2 SS-RSRP measurement accuracy | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB Test 2 Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz Average Ês2 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês2 ±5.65 dB, f <= 40.8 GHz | Test 1 Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 1 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Test 2 Ês1 is NR cell 1 signal Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 7.7.1.2 NR SA FR2-FR2 SS-RSRP measurement accuracy | Noc1 ±5.65 dB, f <= 40.8 GHz Noc1 over measPRBs ±5.65 dB, f <= 40.8 GHz Noc2 ±5.65 dB, f <= 40.8 GHz Noc2 over measPRBs ±5.65 dB, f <= 40.8 GHz Ês1 / Noc1 ±0.3 dB Ês2 / Noc2 ±0.3 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Noc1 is the AWGN on the NR freq 1 Noc2 is the AWGN on the NR freq 2 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 7.7.2.1 NR SA FR2 SS-RSRQ measurement accuracy | Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB, f <= 40.8 GHz Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB | Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 7.7.2.2 NR SA FR2-FR2 SS-RSRQ measurement accuracy | Same as 7.7.1.2 | Same as 7.7.1.2 |
| 7.7.3.1 NR SA FR2 SS-SINR measurement accuracy | Same as 7.7.2.1 | Same as 7.7.2.1 |
| 7.7.3.3 NR SA FR2-FR2 SS-SINR measurement accuracy | Same as 7.7.1.2 | Same as 7.7.1.2 |

Table F.1.1.2-7: Maximum test system uncertainty for RRM requirements for E-UTRA – NR inter-RAT test cases with E-UTRA serving cell

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 8.2.1.1 E-UTRA – NR FR1 cell re-selection to higher priority NR target cell | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Noc2 is the AWGN on cell 2 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB on cell 2 are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 8.2.1.2 E-UTRA – NR FR1 Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16 | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Noc2 is the AWGN on cell 2 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB on cell 2 are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 8.3.1.1 E-UTRA – NR FR1 handover with known target cell | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Noc2 is the AWGN on cell 2 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB on cell 2 are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 8.4.1.1 E-UTRA – NR FR1 SFTD measurement delay in non-DRX | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Noc2 is the AWGN on cell 2 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB on cell 2 are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 8.4.1.2 E-UTRA – NR FR1 SFTD measurement delay in DRX | Same as 8.4.1.1 | Same as 8.4.1.1 |
| 8.4.2.1 E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in non-DRX | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Fading profile ±0.5 dB | Note: Noc1 is the AWGN on cell 1 frequency Noc2 is the AWGN on cell 2 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB on cell 2 are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 8.4.2.2 E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in DRX | Same as 8.4.2.1 | Same as 8.4.2.1 |
| 8.4.2.3 E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in non-DRX | Same as 8.4.2.1 | Same as 8.4.2.1 |
| 8.4.2.4 E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in DRX | Same as 8.4.2.1 | Same as 8.4.2.1 |
| 8.4.2.9 E-UTRA – NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16 | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB |
| 8.5.1.1 E-UTRA – NR FR1 SFTD measurement accuracy | Average Ês1 / Noc1 ±0.8 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Ês1 / Noc1 ±0.8 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB on cell 2 are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 8.5.2.1.1.1 E-UTRA SS-RSRP absolute measurement accuracy of a NR FR1 neighbour cell | Average Noc ±1.5 dB Noc over meas PRBs #RBJ-J+19 ±1.5 dB Average Ês / Noc ±0.3 dB meas PRB Ês1 / Noc1 ±0.8 dB | Noc is the AWGN on the NR freq Ês / Noc is the cell SNR meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 8.5.2.1.2 E-UTRA SS-RSRP absolute measurement accuracy of a NR FR2 neighbour cell | Test 1 Average Noc ±5.65 dB Noc over meas PRBs ±5.65 dB Average Ês / Noc ±0.3 dB Ês / Noc over meas PRBs ±0.3 dB Test 2 Average Es ±5.65 dB Es over meas PRBs ±5.65 dB | Test 1 Noc is the AWGN on the NR freq Ês / Noc is the cell SNR meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 8.5.2.2.1 E-UTRA SS-RSRQ absolute measurement accuracy of a NR FR1 neighbour cell | Same as 8.5.2.1.1.1 | Same as 8.5.2.1.1.1 |
| 8.5.2.2.2 E-UTRA SS-RSRQ absolute measurement accuracy of a NR FR2 neighbour cellA | Same as 8.5.2.1.2 | Same as 8.5.2.1.2 |
| 8.5.2.3.1 E-UTRA SS-SINR absolute measurement accuracy of a NR FR1 neighbour cell | Same as 8.5.2.1.1.1 | Same as 8.5.2.1.1.1 |
| 8.5.2.3.2 E-UTRA SS-SINR absolute measurement accuracy of a NR FR2 neighbour cell | Same as 8.5.2.1.2 | Same as 8.5.2.1.2 |

## F.1.2	Interpretation of measurement results (normative)

See TS 38.521-1 [17] Annex F.2.

## F.1.3	Test Tolerance and Derivation of Test Requirements (informative)

See TS 38.521-1 [17] Annex F.3.

### F.1.3.1	Measurement of test environments

See TS 38.521-1 [17] Annex F.3.1.

### F.1.3.2	Measurement of RRM requirements

Because the relationships between the test system uncertainties and the test tolerances are often complex, it is not always possible to give a simple derivation of the test requirement in this document. The analysis is recorded in 3GPP TR 38 903 [22].

The derivation of the test requirements for the EN-DC FR1 test cases in chapter 4 is defined in Table F.1.3.2-1.

The derivation of the test requirements for the NR SA FR1 test cases in chapter 6 is defined in Table F.1.3.2-2.

The derivation of the test requirements for the EN-DC FR2 test cases in chapter 5 is defined in Table F.1.3.2-3.

The derivation of the test requirements for the NR SA FR2 test cases in chapter 7 is defined in Table F.1.3.2-4.

Table F.1.3.2-1: Derivation of test requirements for EN-DC FR1 RRM tests

| Test | Minimum requirement in TS 38.133 [6] |  | Test tolerance (TT) |  | Test requirement in TS 38.533 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 4.3.2.2.1 | Absolute uplink power: Normal conditions ±9dB Relative uplink power step: Normal conditions ±2.5dB Uplink timing: 15kHz SCS Te ±12*64*Tc 30kHz SCS Te ±8*64*Tc |  | 2.1dB 0.7dB 112Tc 112Tc |  | Absolute uplink power: Normal conditions ±11.1dB Relative uplink power step: Normal conditions ±3.2dB Uplink timing: 15kHz SCS Te ±880*Tc 30kHz SCS Te ±624*Tc |  |
| 4.3.2.2.2 | Same as 4.3.2.2.1 |  | Same as 4.3.2.2.1 |  | Same as 4.3.2.2.1 |  |
| 4.4.1.1 EN-DC FR1 UE transmit timing accuracy | Test 1 (no DRX): Uplink timing: ±12*64 Tc for 15 KHz SSB SCS,15 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,30 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,60 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,15 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,30 kHz UL SCS ±7*64 Tc for 30 KHz SSB SCS,60 kHz UL SCS Max step size Tq: 5.5*64*Tc Min adjust rate Tp: 5.5*64*Tc Max adjust rate: 5.5*64*Tc Ês2 / Noc: +3.00dB Noc = -98 dBm/15 kHz (Config 1,2,3) Test 2 (with DRX): ±12*64 Tc for 15 KHz SSB SCS,15 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,30 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,60 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,15 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,30 kHz UL SCS ±7*64 Tc for 30 KHz SSB SCS,60 kHz UL SCS Ês2 / Noc: +3.00dB |  | ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc +0.5*64Tc -3.6*64*Tc +1.1*64*Tc +0.3 dB +1.5 dB ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc +0.3dB |  | Test 1 (10MHz Ch BW): Uplink timing: ±13.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±8.75*64*Tc Max step size Tq: 6.0*64*Tc Min adjust rate: 1.9*64*Tc Max adjust rate: 6.6*64*Tc Ês2 / Noc: +3.30dB Noc = -98 dBm/15 kHz (Config 1,2,3) +1.5 dB Test 2 (with DRX): Uplink timing: ±13.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±8.75*64*Tc Ês2 / Noc: +3.30dB |  |
| 4.4.3.1 EN-DC FR1 timing advance adjustment accuracy | Noc = -98 dBm/15 kHz (Config 1, 2, 4, 5) Noc = -95 dBm/15 kHz (Config 3, 6) Êsx / Noc = 3 dB UE Timing Advance Adjustment Accuracy for 15kHz SCS = ±256 Tc+ TT UE Timing Advance Adjustment Accuracy for 30kHz SCS = ±256 Tc+ TT |  | 0 0 0 +/- 88 Tc +/- 88 Tc |  | Noc = -98 dBm/15 kHz (Config 1, 2, 4, 5) Noc = -95 dBm/15 kHz (Config 3, 6) Êsx / Noc = 3 dB UE TAAA for 15kHz SCS = ±344 Tc UE TAAA for 30kHz SCS = ±344 Tc |  |
| 4.5.1.1 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | SNR during T1: 1dB T2: -7dB T3: -15dB |  | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB |  | SNR during T1: 1.8dB T2: -6.2dB T3: -15.8dB |  |
| 4.5.1.2 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | SNR during T1: 1dB T2: -7dB T3: -15dB T4: -4.5dB T5: 1dB |  | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: +0.8dB |  | SNR during T1: 1.8dB T2: -6.2dB T3: -15.8dB T4: -5.3dB T5: 1.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in clause D.4.1.1 |  |
| 4.5.1.3 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode | Same as 4.5.1.1 |  | Same as 4.5.1.1 |  | Same as 4.5.1.1 |  |
| 4.5.1.4 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode | Same as 4.5.1.2 |  | Same as 4.5.1.2 |  | Same as 4.5.1.2 |  |
| 4.5.1.5 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | Same as 4.5.1.1 |  | Same as 4.5.1.1 |  | Same as 4.5.1.1 |  |
| 4.5.1.6 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | Same as 4.5.1.2 |  | Same as 4.5.1.2 |  | Same as 4.5.1.2 |  |
| 4.5.1.7 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | Same as 4.5.1.1 |  | Same as 4.5.1.1 |  | Same as 4.5.1.1 |  |
| 4.5.1.8 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | Same as 4.5.1.2 |  | Same as 4.5.1.2 |  | Same as 4.5.1.2 |  |
| 4.5.2.1 EN-DC FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC | SNRs as specified |  | 0.6dB |  | Formula: SNR + TT |  |
| 4.5.2.2 EN-DC FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC | Same as 4.5.2.1 |  | Same as 4.5.2.1 |  | Same as 4.5.2.1 |  |
| 4.5.2.3 EN-DC FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |  | During T1: 0dB 0dB 0dB 0dB |  | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |  |
| 4.5.2.4 EN-DC FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC | Same as 4.5.2.3 |  | Same as 4.5.2.3 |  | Same as 4.5.2.3 |  |
| 4.5.2.5 EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |  | During T1: 0dB 0dB 0dB 0dB |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |  |
| 4.5.2.6 EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC | Same as 4.5.2.5 |  | Same as 4.5.2.5 |  | Same as 4.5.2.5 |  |
| 4.5.3.1 EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T3: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T3: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |  |
| 4.5.3.2 EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 320ms SCell measurement cycle | Same as 4.5.3.1 |  | Same as 4.5.3.1 |  | Same as 4.5.3.1 |  |
| 4.5.3.3 EN-DC FR1 SCell activation and deactivation of unknown SCell in non-DRX | Same as 4.5.3.1 |  | Same as 4.5.3.1 |  | Same as 4.5.3.1 |  |
| 4.5.4.1 EN-DC FR1 UE UL carrier RRC reconfiguration delay | During T1: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB During T2: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB During T3: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  | During T1: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB During T2: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB During T3: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB |  |
| 4.5.5.1 EN-DC FR1 SSB-based beam failure detection and link recovery in non-DRX | q0 SSB SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10dB |  | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 SSB during T1: 0dB -0.2dB q1 SSB during T2: 0dB -0.2dB q1 SSB during T3, T4 and T5: 0dB +0.2dB |  | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in clause D.4.1.1 q1 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10.2dB |  |
| 4.5.5.2 EN-DC FR1 SSB-based beam failure detection and link recovery in DRX | Same as 4.5.5.1 |  | Same as 4.5.5.1 |  | Same as 4.5.5.1 |  |
| 4.5.5.3 EN-DC FR1 CSI-RS-based beam failure detection and link recovery in non-DRX | q0 CSI-RS SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 CSI-RS during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 CSI-RS during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 CSI-RS during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10dB |  | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 CSI-RS during T1: 0dB -0.2dB q1 CSI-RS during T2: 0dB -0.2dB q1 CSI-RS during T3, T4 and T5: 0dB +0.2dB |  | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in clause D.4.1.1 q1 CSI-RS during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 CSI-RS during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 CSI-RS during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10.2dB |  |
| 4.5.5.4 EN-DC FR1 CSI-RS-based beam failure detection and link recovery in DRX | Same as 4.5.5.3 |  | Same as 4.5.5.3 |  | Same as 4.5.5.3 |  |
| 4.5.6.1.1 EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T3: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB |  | During T1: 0dB 0dB During T2: 0dB 0dB During T3: 0dB 0dB |  | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T3: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB |  |
| 4.5.6.1.2 EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB During T3: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB During T3: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB |  |
| 4.5.6.2.1 EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB |  | During T1: 0dB 0dB |  | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB |  |
| 4.5.7.1 EN-DC FR1 addition and release delay of known PSCell |  | For Test Configurations 1,2,4,5 Noc: -88dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -96 dBm/SCS For Test Configurations 3,6 Noc: -88dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -93 dBm/SCS |  | -0.6dB 0dB -3dB -0.6dB 0dB -3dB |  | Noc: -88.6dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -99 dBm/SCS Noc: -88.6dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -96 dBm/SCS |  |
| 4.5.8.1 EN-DC FR1 interruptions at switching between two uplink carriers |  | Noc: -104dBm/15kHz Ês / Noc: 17dB |  | 0dB 0dB |  | Noc: -104dBm/15kHz Ês / Noc: 17dB |  |
| 4.6.1.1 EN-DC FR1 event-triggered reporting without gap in non-DRX | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |  |
| 4.6.1.2 EN-DC FR1 event-triggered reporting without gap in DRX | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  |
| 4.6.1.3 EN-DC FR1 event-triggered reporting with gap in non-DRX | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  |
| 4.6.1.4 EN-DC FR1 event-triggered reporting with gap in DRX | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  |
| 4.6.1.5 EN-DC FR1 event-triggered reporting without gap in non-DRX with SSB time index detection | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  |
| 4.6.1.6 EN-DC FR1 event-triggered reporting with gap in non-DRX with SSB time index detection | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  |
| 4.6.1.7 EN-DC FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  |
| 4.6.2.1 EN-DC FR1-FR1 event-triggered reporting in non-DRX | During T1: Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: -infinity During T2: Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: +7.00dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  | During T1: Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: -infinity During T2: Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: +7.00dB |  |
| 4.6.2.2 EN-DC FR1-FR1 event-triggered reporting in DRX | Same as 4.6.2.1 |  | Same as 4.6.2.1 |  | Same as 4.6.2.1 |  |
| 4.6.2.5 EN-DC FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | Same as 4.6.2.1 |  | Same as 4.6.2.1 |  | Same as 4.6.2.1 |  |
| 4.6.2.6 EN-DC FR1-FR1 event-triggered reporting in DRX with SSB time index detection | Same as 4.6.2.1 |  | Same as 4.6.2.1 |  | Same as 4.6.2.1 |  |
| 4.6.4.1 EN-DC FR1 SSB-based L1-RSRP measurement in non-DRX | During T1: Noc: -94.65dBm/15kHz Ês0 / Noc: +0.00dB Ês1 / Noc: -infinity During T2: Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported SSB#0 SS-RSRP values ± 10dB Reported SSB#1 SS-RSRP values ± 10dB Reported Differential SS-RSRP values ± 3dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.5dB Via mapping |  | During T1: Noc: -94.65dBm/15kHz Ês0 / Noc: +0.00dB Ês1 / Noc: -infinity During T2: Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.50dB For configuration 1,2,4,5 SSB#1: RSRP_55 to RSRP_75 SSB#0: DIFFRSRP_0 to DIFFRSRP_3 For configuration 3,6 SSB#1: RSRP_58 to RSRP_78 SSB#0: DIFFRSRP_0 to DIFFRSRP_3 |  |
| 4.6.4.2 EN-DC FR1 SSB-based L1-RSRP measurement in DRX | Same as 4.6.4.1 |  | Same as 4.6.4.1 |  | Same as 4.6.4.1 |  |
| 4.6.4.3 EN-DC FR1 CSI-RS-based L1-RSRP measurement in non-DRX | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported CSI-RSRP values ± 10dB Reported Differential CSI-RSRP values ± 3dB |  | 0dB 0dB 0.5dB Via mapping |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.50dB For configuration 1,2,4,5 CSI-RS#1: RSRP_55 to RSRP_75 CSI-RS#0: DIFFRSRP_0 to DIFFRSRP_3 For configuration 3,6 CSI-RS#1: RSRP_58 to RSRP_78 CSI-RS#0: DIFFRSRP_0 to DIFFRSRP_3 |  |
| 4.6.4.4 EN-DC FR1 CSI-RS-based L1-RSRP measurement in DRX | Same as 4.6.4.3 |  | Same as 4.6.4.3 |  | Same as 4.6.4.3 |  |
| 4.6.4.5 EN-DC FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16 | Same as 4.6.4.1 |  | Same as 4.6.4.1 |  | Same as 4.6.4.1 |  |
| 4.7.1.1.1 EN-DC FR1 SS-RSRP absolute measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |
|  | Test 1: Noc: -106dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported RSRP values: ±4.5dB Test 2: Noc: -88dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported RSRP values: ±8dB Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: -1.0dB Reported RSRP values: ±6dB |  | Test 1: -1.5dB 0dB +0.4dB Via mapping Test 2: 0dB 0dB +0.4dB Via mapping Test 3: 0dB 0dB +0.2dB Via mapping |  | Test 1: Noc: -107.5dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB RSRP_44 to RSRP_56 Test 2: Noc: -88dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB RSRP_60 to RSRP_79 Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: -0.8dB RSRP_34 to RSRP_46 RSRP_34 to RSRP_46 RSRP_35 to RSRP_47 RSRP_35 to RSRP_47 RSRP_36 to RSRP_48 RSRP_37 to RSRP_49 RSRP_37 to RSRP_49 depending on operating band |  |
|  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |
|  | Test 1: Noc: -110dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported RSRP values: ±4.5dB Test 2: Noc: -91dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported RSRP values: ±8dB Test 3: Noc: -113dBm/30kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: -1.0dB Reported RSRP values: ±6dB |  | Test 1: -0.8dB 0dB +0.4dB Via mapping Test 2: 0dB 0dB +0.4dB Via mapping Test 3: 0dB 0dB +0.2dB Via mapping |  | Test 1: Noc: -110.8dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB RSRP_41 to RSRP_53 Test 2: Noc: -91dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB RSRP_57 to RSRP_76 Test 3: Noc: -113dBm/30kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: -0.8dB RSRP_37 to RSRP_49 RSRP_37 to RSRP_49 RSRP_38 to RSRP_50 RSRP_38 to RSRP_50 RSRP_39 to RSRP_51 RSRP_40 to RSRP_52 RSRP_40 to RSRP_52 depending on operating band |  |
| The derivation of the RSRP values takes into account the uncertainty in Cell 3 SS-RSRP from Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. In all cases the RSRP values are 4.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |
| 4.7.1.1.2 EN-DC FR1 SS-RSRP relative measurement accuracy | Test 1: Noc: Test configuration 1, 2, 4, 5:-106dBm/15kHz Test configuration 3, 6:-110dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported relative SS-RSRP values: ±3dB Test 2: Noc: -88dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported relative SS-RSRP values: ±3dB Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: -1.0dB Reported relative SS-RSRP values: ±3dB |  | Test 1: 0 dB 0 dB 0 dB +1.0dB Via mapping Test 2: 0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB +1.0dB Via mapping |  | Test 1: Noc: -106 dBm/15kHz Noc: -110 dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +2.0dB RSRP_x-8 to RSRP_x-1 Test 2: Noc: -88dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +2.0dB RSRP_x-8 to RSRP_x-1 Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: 0dB RSRP_x-7 to RSRP_x+1 |  |
| The derivation of the SS-RSRP values takes into account the uncertainty in Cell 2 and Cell 3 RSRP from Noc, Ês2 / Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for both normal and extreme conditions. |  |  |  |  |  |  |
| 4.7.1.2.1 EN-DC FR1-FR1 SS-RSRP absolute measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |
|  | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB Reported RSRP values: ±8dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc2: -118dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB Reported RSRP values: ±4.5dB |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -94.65dBm/15kHz Noc2: -94.65dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB RSRP_62 to RSRP_81 Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc2: -118dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB RSRP_32 to RSRP_45 RSRP_33 to RSRP_45 RSRP_33 to RSRP_46 RSRP_34 to RSRP_46 RSRP_34 to RSRP_47 RSRP_35 to RSRP_48 RSRP_36 to RSRP_48 depending on operating band |  |
|  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |
|  | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB Reported RSRP values: ±8dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB Reported RSRP values: ±4.5dB |  | Test 1: -1.35dB -1.35dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -96dBm/15kHz Noc2: -96dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB RSRP_64 to RSRP_83 Test 2: Noc: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB RSRP_35 to RSRP_48 RSRP_36 to RSRP_48 RSRP_36 to RSRP_49 RSRP_37 to RSRP_49 RSRP_37 to RSRP_50 RSRP_38 to RSRP_51 RSRP_39 to RSRP_51 depending on operating band |  |
| The derivation of the RSRP values takes into account the uncertainty in Cell 3 SS-RSRP from Noc2 and Ês3 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. For extreme conditions, the RSRP values are 3.0dB wider at each end for Test 1, and 4.5 dB wider at each for Test 2. |  |  |  |  |  |  |
| 4.7.1.2.2 EN-DC FR1-FR1 SS-RSRP relative measurement accuracy | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB Reported relative RSRP values: ±4.5dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB Reported relative RSRP values: ±4.5dB |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -94.65dBm/15kHz Noc2: -94.65dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB RSRP_x-7 to RSRP_x+7 Test 2: Noc: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB RSRP_x-31 to RSRP_x-18 |  |
| The derivation of the SS-RSRP values takes into account the uncertainty in Cell 2 and Cell 3 RSRP from Noc, Ês2 / Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. For extreme conditions, the RSRP values are 1.5dB wider at each end. |  |  |  |  |  |  |
| 4.7.2.1 EN-DC FR1 SS-RSRQ measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |
|  | Test 1: Noc: -85dBm/15kHz Ês2 / Noc: +3.0dB Ês3 / Noc: +3.0dB Reported RSRQ values: ±2.5dB Test 2: Noc: -101dBm/15kHz Ês2 / Noc: -2.9dB Ês3 / Noc: -2.9dB Reported RSRQ values: ±3.5dB Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês2 / Noc: -4.0dB Ês3 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  | Test 1: -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB Via mapping |  | Test 1: Noc: -86.5dBm/15kHz Ês2 / Noc: +3.0dB Ês3 / Noc: +3.0dB RSRQ_52 to RSRQ_62 Test 2: Noc: -101dBm/15kHz Ês2 / Noc: -2.9dB Ês3 / Noc: -2.9dB RSRQ_46 to RSRQ_60 Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês2 / Noc: -4.0dB Ês3 / Noc: -4.0dB RSRQ_44 to RSRQ_59 |  |
|  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |
|  | Test 1: Noc: -91dBm/15kHz Ês2 / Noc: +3.0dB Ês3 / Noc: +3.0dB Reported RSRQ values: ±2.5dB Test 2: N/A Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês2 / Noc: -4.0dB Ês3 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  | Test 1: -1.6dB 0dB 0dB Via mapping Test 2: N/A Test 3: 0dB 0dB 0dB Via mapping |  | Test 1: Noc: -92.6dBm/15kHz Ês2 / Noc: +3.0dB Ês3 / Noc: +3.0dB RSRQ_52 to RSRQ_62 Test 2: N/A Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês2 / Noc: -4.0dB Ês3 / Noc: -4.0dB RSRQ_44 to RSRQ_59 |  |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 3 SS-RSRQ from Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the SS-RSRQ values are 1.5dB wider at each end for Test 1, and 0.5 dB wider at each for Tests 2 and 3. |  |  |  |  |  |  |
| 4.7.2.2.1 EN-DC FR1-FR1 SS-RSRQ absolute measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |
|  | Test 1: Noc1: -80.18 dBm/15kHz Noc2: -80.18 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB |  | Test 1: -1.5dB -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -81.68dBm/15kHz Noc2: -81.68dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 |  |
|  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |
|  | Test 1: Noc1: -86.27 dBm/15kHz Noc2: -86.27 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB |  | Test 1: -1.53dB -1.53dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -87.8dBm/15kHz Noc2: -87.8dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 |  |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 3 SS-RSRQ from Noc2 and Ês3 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the RSRQ values are 1.5dB wider at each end. |  |  |  |  |  |  |
| 4.7.2.2.2 EN-DC FR1-FR1 SS-RSRQ relative measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |
|  | Test 1: Noc1: -80.18 dBm/15kHz Noc2: -80.18 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±3dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±3dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±3dB |  | Test 1: -1.5dB -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -81.68dBm/15kHz Noc2: -81.68dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB RSRQ_x-11 to RSRQ_x+2 |  |
|  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |
|  | Test 1: Noc1: -86.27 dBm/15kHz Noc2: -86.27 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±3dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±3dB |  | Test 1: -1.53dB -1.53dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping |  | Test 1: Noc1: -87.8dBm/15kHz Noc2: -87.8dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB RSRQ_x-11 to RSRQ_x+2 |  |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 2 and Cell 3 SS-RSRQ from Noc2 and Ês3 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the RSRQ values are 1dB wider at each end. |  |  |  |  |  |  |
| 4.7.3.1 EN-DC FR1 SS-SINR measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |
|  | Test 1: Noc: -93dBm/15kHz Ês2 / Noc: +4.54dB Ês3 / Noc: +4.54dB Reported SINR values: ±3.5dB Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês2 / Noc: -4dB Ês3 / Noc: -4dB Reported SINR values: ±3.5dB |  | Test 1: 0dB 0dB 0dB Via mapping Test 2: 0dB 0.5dB 0.5dB Via mapping |  | Test 1: Noc: -93dBm/15kHz Ês2 / Noc: +4.54dB Ês3 / Noc: +4.54dB SINR_31 to SINR_49 Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês2 / Noc: -3.5dB Ês3 / Noc: -3.5dB SINR_28 to SINR_45 |  |
|  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |
|  | Test 1: Noc: -93dBm/15kHz Ês2 / Noc: +4.54dB Ês3 / Noc: +4.54dB Reported SINR values: ±3.5dB Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês2 / Noc: -4dB Ês3 / Noc: -4dB Reported SINR values: ±3.5dB |  | Test 1: -0.2dB 0dB 0dB Via mapping Test 2: 0dB 0.5dB 0.5dB Via mapping |  | Test 1: Noc: -93.2dBm/15kHz Ês2 / Noc: +4.54dB Ês3 / Noc: +4.54dB SINR_31 to SINR_49 Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês2 / Noc: -3.5dB Ês3 / Noc: -3.5dB SINR_28 to SINR_45 |  |
| The derivation of the SINR values takes into account the uncertainty in Cell 3 SS- SINR from Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 0.5dB wider at each end. |  |  |  |  |  |  |
| 4.7.3.2.1 EN-DC FR1-FR1 SS-SINR absolute measurement accuracy | TEST CONFIGURATION 1, 2, 3, 4, 5, 6 |  |  |  |  |  |
|  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported SINR values: ±3dB Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês2 / Noc1: 20dB Ês3 / Noc2: 20dB Reported SINR values: ±3dB Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -4dB Ês3 / Noc2: -4dB Reported SINR values: ±3.5dB |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0.8dB 0.8dB Via mapping |  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB SINR_35 to SINR_51 Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês2 / Noc1: 20dB Ês3 / Noc2: 20dB SINR_79 to SINR_94 Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3.2dB Ês3 / Noc2: -3.2dB SINR_32 to SINR_49 |  |
| The derivation of the SINR values takes into account the uncertainty in Cell 3 SS- SINR from Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 1dB wider at each for Tests 1 and 2 and 0.5dB wider at each end for Test 3. |  |  |  |  |  |  |
| 4.7.3.2.2 EN-DC FR1-FR1 SS-SINR relative measurement accuracy | TEST CONFIGURATION 1, 2, 3, 4, 5, 6 |  |  |  |  |  |
|  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported SINR values: ±3.5dB Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês2 / Noc1: 20dB Ês3 / Noc2: 20dB Reported SINR values: ±3.5dB Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -4dB Ês3 / Noc2: -4dB Reported SINR values: ±4dB |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0.8dB 0.8dB Via mapping |  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB SINR_x-10 to SINR_x+10 Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês2 / Noc1: 20dB Ês3 / Noc2: 20dB SINR_x-10 to SINR_x+10 Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3.2dB Ês3 / Noc2: -3.2dB SINR_x-11 to SINR_x+11 |  |
| The derivation of the SINR values takes into account the uncertainty in Cell 3 SS- SINR from Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 0.5dB wider at each for Tests 1 and 2 and 0dB wider at each end for Test 3. |  |  |  |  |  |  |
| 4.7.4.1.1 EN-DC FR1 SSB based L1-RSRP absolute measurement accuracy |  |  |  |  |  |  |
| Test Configuration 1,2,4,5 | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±4.5dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±3dB additionally for extreme conditions) |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  | Test 1: Noc1: -94.65dBm/15kHz Ês2 / Noc1: 10dB RSRP_62 to RSRP_82 Test 2: Noc1: -119.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB RSRP_31 to RSRP_44 RSRP_31 to RSRP_45 RSRP_32 to RSRP_45 RSRP_32 to RSRP_46 RSRP_33 to RSRP_46 RSRP_34 to RSRP_47 RSRP_34 to RSRP_48 depending on operating band |  |
| Test Configuration 3,6 | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±4.5dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±3dB additionally for extreme conditions) |  | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  | Test 1: Noc1: -96dBm/15kHz Ês2 / Noc1: 10dB RSRP_63 to RSRP_84 Test 2: Noc1: -116.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB RSRP_34 to RSRP_47 RSRP_34 to RSRP_48 RSRP_35 to RSRP_48 RSRP_35 to RSRP_49 RSRP_36 to RSRP_49 RSRP_37 to RSRP_50 RSRP_37 to RSRP_51 depending on operating band |  |
| 4.7.4.1.2 EN-DC FR1 SSB based L1-RSRP relative measurement accuracy |  |  |  |  |  |  |
| Test Configuration 1,2,4,5 | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±1dB additionally for extreme conditions) |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  | Test 1: Noc1: -94.65dBm/15kHz Ês2 / Noc1: 10dB RSRP_x-3 to RSRP_x+3 Test 2: Noc1: -119.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB RSRP_x-3 to RSRP_x+3 |  |
| Test Configuration 3,6 | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±1dB additionally for extreme conditions) |  | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  | Test 1: Noc1: -96dBm/15kHz Ês2 / Noc1: 10dB RSRP_x-3 to RSRP_x+3 Test 2: Noc1: -116.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB RSRP_x-3 to RSRP_x+3 |  |
| 4.7.4.2.1 EN-DC FR1 CSI-RS based L1-RSRP absolute measurement accuracy | Same as 4.7.4.1.1 |  | Same as 4.7.4.1.1 |  | Same as 4.7.4.1.1 |  |
| 4.7.4.2.2 EN-DC FR1 CSI-RS based L1-RSRP relative measurement accuracy | Same as 4.7.4.1.2 |  | Same as 4.7.4.1.2 |  | Same as 4.7.4.1.2 |  |
| 4.7.5.1 EN-DC FR1 SFTD measurement accuracy | Noc1: -104dBm/15kHz Ês1 / Noc1: -3.0dB Noc2: -104dBm/15kHz Ês2 / Noc2: 17.0dB |  | 0dB 0.3dB 0dB 0dB |  | Noc1: -104dBm/15kHz Ês1 / Noc1: -2.7dB Noc2: -104dBm/15kHz Ês2 / Noc2: 17.0dB |  |

Table F.1.3.2-2: Derivation of test requirements for NR SA FR1 RRM tests

| Test |  | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- | --- |
| 6.1.1.1 NR SA FR1 cell re-selection |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +13dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.45dB During T3: 0dB 0.45dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16.45dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +16.45dB Ês2 / Noc: +13dB |
| 6.1.1.2 NR SA FR1-FR1 cell re-selection |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: +12dB | During T1: 0dB -2dB 1.6dB 0.4dB During T2: 0dB 0dB 1.6dB 0dB During T3: 0dB 0dB 1.6dB 1.6dB | During T1: Noc1: -98dBm/15kHz Noc2: -100dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -3.6dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: 13.6dB |
| 6.1.1.3 NR SA FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +13dB In Signalling SSearchDeltaP = 3dB | During T1: 0dB -0.45dB 0dB During T2: 0dB 0dB -0.45dB In Signalling 3dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +12.55dB Ês2 / Noc: +16dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +12.55dB In Signalling SSearchDeltaP = 6dB |
| 6.1.1.4 NR SA FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +13dB | During T1: 0dB -0.45dB 0dB During T2: 0dB 0dB -0.45dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +12.55dB Ês2 / Noc: +16dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +12.55dB |
| 6.1.1.5 NR SA FR1-FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: -4dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: +12dB In signalling: Thresh_X_HighP of Cell 1 = 48dB Thresh_X_HighP of Cell 2 = 48dB Thresh_X_LowP of Cell 1 = 50dB Thresh_X_LowP of Cell 2 = 50dB Thresh_Serving_LowP of Cell 1 = 44dB Thresh_Serving_LowP of Cell 2 = 44dB SSearchDeltaP of Cell 1 = 3dB SSearchDeltaP of Cell 2 = 3dB | During T1: 0dB 0dB 0dB 0.3dB During T2: 0dB 0dB 0dB -2.25dB In signalling: 0dB -4dB -2dB 0dB 0dB 2dB 0dB 12dB | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: -3.7dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc1: -98dBm/15kHz Ês2 / Noc2: +9.75dB In signalling: Thresh_X_HighP of Cell 1 = 48dB Thresh_X_HighP of Cell 2 = 44dB Thresh_X_LowP of Cell 1 = 48dB Thresh_X_LowP of Cell 2 = 50dB Thresh_Serving_LowP of Cell 1 = 44dB Thresh_Serving_LowP of Cell 2 = 46dB SSearchDeltaP of Cell 1 = 3dB SSearchDeltaP of Cell 2 = 15dB |
| 6.1.1.6 NR SA FR1-FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion |  | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: -4dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc2: -98dBm/15kHz Ês2 / Noc2: +12dB In signalling: SSearchThresholdP of Cell 1 = 50dB SSearchThresholdP of Cell 2 = 50dB | During T1: 0dB 2dB -2dB 0.35dB During T2: 0dB 0dB -2dB 4dB In signalling: 0dB -16dB | During T1: Noc1: -98dBm/15kHz Ês1 / Noc1: +16dB Noc2: -100dBm/15kHz Ês2 / Noc2: -3.65dB During T2: Noc1: -98dBm/15kHz Ês1 / Noc1: +14dB Noc1: -100dBm/15kHz Ês2 / Noc2: +16dB In signalling: SSearchThresholdP of Cell 1 = 50dB SSearchThresholdP of Cell 2 = 34dB |
| 6.1.2.1 NR SA FR1 – E-UTRA cell re-selection to higher priority E-UTRA |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: +12dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 1.6dB 1.6dB During T3: 0dB -2dB 1.6dB 0.4dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: 13.6dB During T3: Noc1: -98dBm/15kHz Noc2: -100dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -3.6dB |
| 6.1.2.2 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: +14dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +12dB Ês2 / Noc2: +14dB | During T1: -2dB 0dB 0.4dB 1.6dB During T2: 0dB 0dB 1.6dB 0dB | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -3.6dB Ês2 / Noc2: +15.6dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +13.6dB Ês2 / Noc2: +14dB |
| 6.1.2.3 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: +14dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +12dB Ês2 / Noc2: +14dB | During T1: -2dB 0dB 0.4dB 1.6dB During T2: 0dB 0dB 1.6dB 0dB | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -3.6dB Ês2 / Noc2: +15.6dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +13.6dB Ês2 / Noc2: +14dB |
| 6.1.2.4 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: +14dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +12dB Ês2 / Noc2: +14dB | During T1: -2dB 0dB 0.4dB 1.6dB During T2: 0dB 0dB 1.6dB 0dB | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -3.6dB Ês2 / Noc2: +15.6dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +13.6dB Ês2 / Noc2: +14dB |
| 6.1.2.5 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA for UE configured with highSpeedMeasFlag-r16 |  | Same as 6.1.2.2 | Same as 6.1.2.2 | Same as 6.1.2.2 |
| 6.3.1.1 NR SA FR1 handover with known target cell |  | TEST CONFIGURATION 1, 2 A3-Offset: 0dB | -1dB | A3-Offset: -1dB |
|  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +11dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +11dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB -1dB During T3: 0dB 0dB 0dB -1dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +10dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +10dB |
|  |  | TEST CONFIGURATION 3 A3-Offset: 0dB | -1dB | A3-Offset: -1dB |
|  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +11dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +11dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB -1dB During T3: 0dB 0dB 0dB -1dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +10dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +10dB |
| 6.3.1.2 NR SA FR1 handover with unknown target cell |  | A3-Offset: 0dB During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +8dB | -1dB During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 1.5dB | A3-Offset: -1dB During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +9.5dB |
| 6.3.1.3 NR SA FR1-FR1 Handover with unknown Target Cell |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +5dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 1.7dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +6.7dB |
| 6.3.1.4 NR SA FR1 – E-UTRA handover with known target cell |  | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +12dB Ês2 / Noc2: -infinity During T2: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: +8dB During T3: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: +8dB | During T1: 0dB 0dB 1.55dB 0dB During T2: 0dB 0dB -1.55dB 1.55dB During T3: 0dB 0dB -1.55dB 1.55dB | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +13.55dB Ês2 / Noc2: -infinity During T2: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -5.55dB Ês2 / Noc2: +9.55dB During T3: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -5.55dB Ês2 / Noc2: +9.55dB |
| 6.3.1.5 NR SA FR1 – E-UTRA handover with unknown target cell |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: +7dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: +7dB |
| 6.3.1.6 NR SA FR1 – UTRAN FDD handover with known target cell |  | During T1: Noc: -100dBm/15kHz Ês / Noc: 12dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB During T2: Noc: -100dBm/15kHz Ês / Noc: -4dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB During T3: Noc: -100dBm/15kHz Ês / Noc: -4dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB | During T1: -1.54dB 0dB 0dB 0dB 0dB 0dB During T2: -1.54dB 0dB 0dB 0dB 0dB 0dB During T3: -1.54dB 0dB 0dB 0dB 0dB 0dB | During T1: Noc: -101.54dBm/15kHz Ês / Noc: 12dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB During T2: Noc: -101.54dBm/15kHz Ês / Noc: -4dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB During T3: Noc: -101.54dBm/15kHz Ês / Noc: -4dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB |
| 6.3.1.7 NR SA FR1 synchronous DAPS handover |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB In signaling: A3-offset: 0dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 1dB During T3: 0dB 0dB 1dB During T4: 0dB 0dB 1dB During T5: 0dB 0dB 1dB In signaling: -1dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB In signaling: A3-offset: -1dB |
| 6.3.1.8 NR SA FR1 asynchronous DAPS handover |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +9dB In signaling: A3-offset: 0dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 1dB During T3: 0dB 0dB 1dB During T4: 0dB 0dB 1dB During T5: 0dB 0dB 1dB In signaling: -1dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB In signaling: A3-offset: -1dB |
| 6.3.1.9 NR SA FR1 Intra-band inter-frequency synchronous DAPS handover |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB In signaling: A3-offset: 0dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 3.7dB During T3: 0dB 0dB 3.7dB During T4: 0dB 0dB 3.7dB During T5: 0dB 0dB 3.7dB In signaling: -3dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB In signaling: A3-offset: -3dB |
| 6.3.1.10 NR SA FR1 Intra-band inter-frequency asynchronous DAPS handover |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +8dB In signaling: A3-offset: 0dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 3.7dB During T3: 0dB 0dB 3.7dB During T4: 0dB 0dB 3.7dB During T5: 0dB 0dB 3.7dB In signaling: -3dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB During T4: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB During T5: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11.7dB In signaling: A3-offset: -3dB |
| 6.3.2.1.1 NR SA FR1 RRC re-establishment |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +7dB Ês2 / Noc: +4dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +7dB Ês2 / Noc: +4dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB |
| 6.3.2.1.2 NR SA FR1 - FR1 RRC re-establishment |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: +7dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: +7dB |
| 6.3.2.1.3 NR SA FR1 RRC re-establishment without serving cell timing |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: -infinity During T3: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: -infinity During T3: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB |
| 6.3.2.2.1 |  | Absolute uplink power: Normal conditions ±9dB Relative uplink power step: Normal conditions ±2.5dB Uplink timing: 15kHz SCS Te ±12*64*Tc 30kHz SCS Te ±8*64*Tc | 2.1dB 0.7dB 112Tc 112Tc | Absolute uplink power: Normal conditions ±11.1dB Relative uplink power step: Normal conditions ±3.2dB Uplink timing: 15kHz SCS Te ±880*Tc 30kHz SCS Te ±624*Tc |
| 6.3.2.2.2 |  | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 | Same as 6.3.2.2.1 |
| 6.3.2.3.1 NR SA FR1 RRC connection release with redirection |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB |
| 6.3.2.3.2 NR SA FR1 – E-UTRA RRC connection release with redirection |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB |
| 6.3.3.1 NR SA FR1 conditional handover |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +11dB In signaling: A3-offset: 0dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB -1dB In signaling: -1dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +8dB Ês2 / Noc: +10dB In signaling: A3-offset: -1dB |
| 6.3.3.2 NR SA FR1-FR1 conditional handover |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +5dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 1.7dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +6.7dB |
| 6.4.1.1 EN-DC FR1 UE transmit timing accuracy |  | Test 1 (no DRX): Uplink timing: ±12*64 Tc for 15 KHz SSB SCS,15 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,30 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,60 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,15 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,30 kHz UL SCS ±7*64 Tc for 30 KHz SSB SCS,60 kHz UL SCS Max step size Tq: 5.5*64*Tc Min adjust rate Tp: 5.5*64*Tc Max adjust rate: 5.5*64*Tc Ês2 / Noc: +3.00dB Noc = -98 dBm/15 kHz (Config 1,2,3) Test 2 (with DRX): ±12*64 Tc for 15 KHz SSB SCS,15 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,30 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,60 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,15 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,30 kHz UL SCS ±7*64 Tc for 30 KHz SSB SCS,60 kHz UL SCS Ês1 / Noc: +3.00dB | ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc +0.5*64Tc -3.6*64*Tc +1.1*64*Tc +0.3 dB +1.5 dB ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc +0.3dB | Test 1 (10MHz Ch BW): Uplink timing: ±13.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±8.75*64*Tc Max step size Tq: 6.0*64*Tc Min adjust rate: 1.9*64*Tc Max adjust rate: 6.6*64*Tc Ês1 / Noc: +3.30dB Noc = -98 dBm/15 kHz (Config 1,2,3) +1.5 dB Test 2 (with DRX): Uplink timing: ±13.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±8.75*64*Tc Ês1 / Noc: +3.30dB |
| 6.4.3.1 NR SA FR1 timing advance adjustment accuracy |  | Noc = -98 dBm/15 kHz (Config 1, 2, 4, 5) Noc = -95 dBm/15 kHz (Config 3, 6) Êsx / Noc = 3 dB UE Timing Advance Adjustment Accuracy for 15kHz SCS = ±256 Tc+ TT UE Timing Advance Adjustment Accuracy for 30kHz SCS = ±256 Tc+ TT | 0 0 0 +/- 88 Tc +/- 88 Tc | Noc = -98 dBm/15 kHz (Config 1, 2) Noc = -95 dBm/15 kHz (Config 3) Êsx / Noc = 3 dB UE TAAA for 15kHz SCS = ±344 Tc UE TAAA for 30kHz SCS = ±344 Tc |
| 6.5.1.1 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode |  | SNR during T1: 1dB T2: -7dB T3: -15dB | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB | SNR during T1: 1.8dB T2: -6.2dB T3: -15.8dB |
| 6.5.1.2 NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode |  | SNR during T1: 1dB T2: -7dB T3: -15dB T4: -4.5dB T5: 1dB | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: +0.8dB | SNR during T1: 1.8dB T2: -6.2dB T3: -15.8dB T4: -5.3dB T5: 1.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in clause D.4.1.1 |
| 6.5.1.3 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode |  | Same as 6.5.1.1 | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.1.4 NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode |  | Same as 6.5.1.2 | Same as 6.5.1.2 | Same as 6.5.1.2 |
| 6.5.1.5 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode |  | Same as 6.5.1.1 | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.1.6 NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode |  | Same as 6.5.1.2 | Same as 6.5.1.2 | Same as 6.5.1.2 |
| 6.5.1.7 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode |  | Same as 6.5.1.1 | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.1.8 NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode |  | Same as 6.5.1.2 | Same as 6.5.1.2 | Same as 6.5.1.2 |
| 6.5.2.1 NR SA FR1 interruptions during measurements on deactivated NR SCC |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB | During T1: 0dB 0dB 0dB 0dB | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB |
| 6.5.3.1 NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T3: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T3: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB |
| 6.5.3.2 NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 320ms SCell measurement cycle |  | Same as 6.5.3.1 | Same as 6.5.3.1 | Same as 6.5.3.1 |
| 6.5.3.3 NR SA FR1 SCell activation and deactivation of unknown SCell in non-DRX |  | Same as 6.5.3.1 | Same as 6.5.3.1 | Same as 6.5.3.1 |
| 6.5.5.1 NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX |  | q0 SSB SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 SSB during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1 SSB during T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1 SSB during T3, T4 and T5: Noc1: -98dBm/15kHz Ês1 / Noc1: +10dB | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 SSB during T1: 0dB -0.2dB q1 SSB during T2: 0dB -0.2dB q1 SSB during T3, T4 and T5: 0dB +0.2dB | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in clause D.4.1.1 q1 SSB during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1 SSB during T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1 SSB during T3, T4 and T5: Noc1: -98dBm/15kHz Ês1 / Noc1: +10.2dB |
| 6.5.5.2 NR SA FR1 SSB-based beam failure detection and link recovery in DRX |  | Same as 6.5.5.1 | Same as 6.5.5.1 | Same as 6.5.5.1 |
| 6.5.5.3 NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX |  | q0 CSI-RS SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 CSI-RS during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1 CSI-RS during T2: Noc2: -98dBm/15kHz Ês1 / Noc1: -10dB q1 CSI-RS during T3, T4 and T5: Noc2: -98dBm/15kHz Ês1 / Noc1: +10dB | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 CSI-RS during T1: 0dB -0.2dB q1 CSI-RS during T2: 0dB -0.2dB q1 CSI-RS during T3, T4 and T5: 0dB +0.2dB | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in clause D.4.1.1 q1 CSI-RS during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1 CSI-RS during T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1 CSI-RS during T3, T4 and T5: Noc1: -98dBm/15kHz Ês1 / Noc1: +10.2dB |
| 6.5.5.4 NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX |  | Same as 6.5.5.3 | Same as 6.5.5.3 | Same as 6.5.5.3 |
| 6.5.6.1.1 NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T3: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T3: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB |
| 6.5.6.1.2 NR SA FR1 DCI-based DL active BWP switch in non-DRX |  | During T1: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB During T2: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB During T3: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB | During T1: 0dB 0dB During T2: 0dB 0dB During T3: 0dB 0dB | During T1: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB During T2: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB During T3: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB |
| 6.5.6.2.1 NR SA FR1 RRC-based DL active BWP switch in non-DRX |  | During T1: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB | During T1: 0dB 0dB | During T1: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB |
| 6.6.1.1 SA event triggered reporting tests without gap under non-DRX |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |
| 6.6.1.2 SA event triggered reporting tests without gap under DRX |  | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.3 SA event triggered reporting tests with per-UE gaps under non-DRX |  | Same as 64.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.4 SA event triggered reporting tests with per-UE gaps under DRX |  | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.5 SA event triggered reporting tests without gap under non-DRX with SSB index reading |  | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.6 SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading |  | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.6.1.7 NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 |  | Same as 6.6.1.1 | Same as 6.6.1.1 | Same as 6.6.1.1 |
| 6.5.3.1 |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T3: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T3: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB |
| 6.5.3.2 |  | Same as 6.5.3.1 | Same as 6.5.3.1 | Same as 6.5.3.1 |
| 6.5.3.3 |  | Same as 6.5.3.1 | Same as 6.5.3.1 | Same as 6.5.3.1 |
| 6.6.2.1 NR SA FR1-FR1 event-triggered reporting in non-DRX |  | During T1: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +7.00dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +7.00dB |
| 6.6.2.2 NR SA FR1-FR1 event-triggered reporting in DRX |  | Same as 6.6.2.1 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.2.5 NR SA FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection |  | Same as 6.6.2.1 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.2.6 NR SA FR1-FR1 event-triggered reporting in DRX with SSB time index detection |  | Same as 6.6.2.1 | Same as 6.6.2.1 | Same as 6.6.2.1 |
| 6.6.3.1 NR SA FR1 – E-UTRAN event-triggered reporting in non-DRX |  | During T1: Freq 1 Noc: -106dBm/15kHz Freq 2 Noc: -106dBm/15kHz Ês1 / Noc: +18.00dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -106dBm/15kHz Freq 2 Noc: -106dBm/15kHz Ês1 / Noc: -2dB Ês2 / Noc: +19.00dB | During T1: 0dB 0dB +1.65dB 0dB During T2: 0dB 0dB -1.65dB 1.65dB | During T1: Freq 1 Noc: -106dBm/15kHz Freq 2 Noc: -106dBm/15kHz Ês1 / Noc: +19.65dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -106dBm/15kHz Freq 2 Noc: -106dBm/15kHz Ês1 / Noc: -3.65dB Ês2 / Noc: +20.65dB |
| 6.6.3.2 NR SA FR1 – E-UTRAN event-triggered reporting in DRX |  | Same as 6.6.3.1 | Same as 6.6.3.1 | Same as 6.6.3.1 |
| 6.6.3.3 NR SA FR1 – E-UTRAN event-triggered reporting in DRX for UE configured with highSpeedMeasFlag-r16 |  | During T1: Freq 1 Noc: -106dBm/15kHz Freq 2 Noc: -106dBm/15kHz Ês1 / Noc: +18.00dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -106dBm/15kHz Freq 2 Noc: -106dBm/15kHz Ês1 / Noc: -2dB Ês2 / Noc: +19.00dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Freq 1 Noc: -106dBm/15kHz Freq 2 Noc: -106dBm/15kHz Ês1 / Noc: +18.00dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -106dBm/15kHz Freq 2 Noc: -106dBm/15kHz Ês1 / Noc: -2dB Ês2 / Noc: +19.00dB |
| 6.6.4.1 NR SA FR1 SSB-based L1-RSRP measurement in non-DRX |  | During T1: Noc: -98dBm/15kHz Ês0 / Noc: +0.00dB Ês1 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported SSB#0 SS-RSRP values ± 10dB Reported SSB#1 SS-RSRP values ± 10dB Reported Differential SS-RSRP values ± 3dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.5dB Via mapping | During T1: Noc: -94.65dBm/15kHz Ês0 / Noc: +0.00dB Ês1 / Noc: -infinity During T2: Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.50dB For configuration 1,2,4,5 SSB#1: RSRP_55 to RSRP_75 SSB#0: DIFFRSRP_0 to DIFFRSRP_3 For configuration 3,6 SSB#1: RSRP_58 to RSRP_78 SSB#0: DIFFRSRP_0 to DIFFRSRP_3 |
| 6.6.4.2 NR SA FR1 SSB-based L1-RSRP measurement in DRX |  | Same as 6.6.4.1 | Same as 6.6.4.1 | Same as 6.6.4.1 |
| 6.6.4.3 NR SA FR1 CSI-RS-based L1-RSRP measurement in non-DRX |  | Noc: -98dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported CSI-RSRP values ± 10dB Reported Differential CSI-RSRP values ± 3dB | 0dB 0dB 0.5dB Via mapping | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.50dB For configuration 1,2,4,5 CSI-RS#1: RSRP_55 to RSRP_75 CSI-RS#0: DIFFRSRP_0 to DIFFRSRP_3 For configuration 3,6 CSI-RS#1: RSRP_58 to RSRP_78 CSI-RS#0: DIFFRSRP_0 to DIFFRSRP_3 |
| 6.6.4.4 NR SA FR1 CSI-RS-based L1-RSRP measurement in DRX |  | Same as 6.6.4.3 | Same as 6.6.4.3 | Same as 6.6.4.3 |
| 6.6.4.5 NR SA FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16 |  | Same as 6.6.4.1 | Same as 6.6.4.1 | Same as 6.6.4.1 |
| 6.6.5.1 NR SA FR1 – UTRAN event-triggered reporting in non-DRX |  | During T1: Noc: -106dBm/15kHz Ês / Noc: 18dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB During T2: Noc: -106dBm/15kHz Ês / Noc: -2dB Ioc: -70dBm/3.84MHz Îor / Ioc: -1.8dB CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB | During T1: 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB 0dB | During T1: Noc: -106dBm/15kHz Ês / Noc: 18dB Ioc: -70dBm/3.84MHz Îor / Ioc: -infinity CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB During T2: Noc: -106dBm/15kHz Ês / Noc: -2dB Ioc: -70dBm/3.84MHz Îor / Ioc: -1.8dB CPICH Ec / Ior: -10dB SCH Ec / Ior: -12dB |
| 6.7.1.1.1 NR SA FR1 SS-RSRP absolute measurement accuracy |  | TEST CONFIGURATION 1, 2, |  |  |
|  |  | Test 1: Noc: -106dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±4.5dB Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±8dB Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported RSRP values: ±6dB | Test 1: -1.5dB 0dB +0.4dB Via mapping Test 2: 0dB 0dB +0.4dB Via mapping Test 3: 0dB 0dB +0.2dB Via mapping | Test 1: Noc: -107.5dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB RSRP_44 to RSRP_56 Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB RSRP_60 to RSRP_79 Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -0.8dB RSRP_34 to RSRP_46 RSRP_34 to RSRP_46 RSRP_35 to RSRP_47 RSRP_35 to RSRP_47 RSRP_36 to RSRP_48 RSRP_37 to RSRP_49 RSRP_37 to RSRP_49 depending on operating band |
|  |  | TEST CONFIGURATION 3 |  |  |
|  |  | Test 1: Noc: -110dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±4.5dB Test 2: Noc: -91dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±8dB Test 3: Noc: -113dBm/30kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported RSRP values: ±6dB | Test 1: -0.8dB 0dB +0.4dB Via mapping Test 2: 0dB 0dB +0.4dB Via mapping Test 3: 0dB 0dB +0.2dB Via mapping | Test 1: Noc: -110.8dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB RSRP_41 to RSRP_53 Test 2: Noc: -91dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB RSRP_57 to RSRP_76 Test 3: Noc: -113dBm/30kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -0.8dB RSRP_37 to RSRP_49 RSRP_37 to RSRP_49 RSRP_38 to RSRP_50 RSRP_38 to RSRP_50 RSRP_39 to RSRP_51 RSRP_40 to RSRP_52 RSRP_40 to RSRP_52 depending on operating band |
| The derivation of the RSRP values takes into account the uncertainty in Cell 2 SS-RSRP from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. In all cases the RSRP values are 4.5dB wider at each end for extreme conditions. |  |  |  |  |
| 6.7.1.1.2 NR SA FR1 SS-RSRP relative measurement accuracy |  | Test 1: Noc: Test configuration 1, 2:-106dBm/15kHz Test configuration 3:-110dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported rel. SS-RSRP values: ±3dB Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported rel. SS-RSRP values: ±3dB Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported re. SS-RSRP values: ±3dB | Test 1: 0 dB 0 dB 0 dB +1.0dB Via mapping Test 2: 0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB +1.0dB Via mapping | Test 1: Noc: -106 dBm/15kHz Noc: -110 dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_x-8 to RSRP_x-1 Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_x-8 to RSRP_x-1 Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: 0dB RSRP_x-7 to RSRP_x+1 |
| The derivation of the SS-RSRP values takes into account the uncertainty in Cell 1 and Cell 2 RSRP from Noc, Ês1 / Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for both normal and extreme conditions. |  |  |  |  |
| 6.7.1.2.1 NR SA FR1-FR1 SS-RSRP absolute measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |
|  | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB Reported RSRP values: ±8dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc2: -118dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB Reported RSRP values: ±4.5dB |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -94.65dBm/15kHz Noc2: -94.65dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB RSRP_62 to RSRP_81 Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc2: -118dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB RSRP_32 to RSRP_45 RSRP_33 to RSRP_45 RSRP_33 to RSRP_46 RSRP_34 to RSRP_46 RSRP_34 to RSRP_47 RSRP_35 to RSRP_48 RSRP_36 to RSRP_48 depending on operating band |
|  | TEST CONFIGURATION 3, 6 |  |  |  |
|  | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB Reported RSRP values: ±8dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB Reported RSRP values: ±4.5dB |  | Test 1: -1.35dB -1.35dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -96dBm/15kHz Noc2: -96dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB RSRP_64 to RSRP_83 Test 2: Noc: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB RSRP_35 to RSRP_48 RSRP_36 to RSRP_48 RSRP_36 to RSRP_49 RSRP_37 to RSRP_49 RSRP_37 to RSRP_50 RSRP_38 to RSRP_51 RSRP_39 to RSRP_51 depending on operating band |
| The derivation of the RSRP values takes into account the uncertainty in Cell 2 SS-RSRP from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. For extreme conditions, the RSRP values are 3.0 dB wider at each end for Test 1, and 4.5 dB wider at each for Test 2. |  |  |  |  |
| 6.7.1.2.2 NR SA FR1-FR1 SS-RSRP relative measurement accuracy | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB Reported relative RSRP values: ±4.5dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB Reported relative RSRP values: ±4.5dB |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -94.65dBm/15kHz Noc2: -94.65dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB RSRP_x-7 to RSRP_x+7 Test 2: Noc: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB RSRP_x-31 to RSRP_x-18 |
| The derivation of the SS-RSRP values takes into account the uncertainty in Cell 1 and Cell 2 RSRP from Noc, Ês1 / Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. For extreme conditions, the RSRP values are 1.5dB wider at each end. |  |  |  |  |
| 6.7.2.1 NR SA FR1 SS-RSRQ measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |
|  | Test 1: Noc: -85dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported RSRQ values: ±2.5dB Test 2: Noc: -101dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB Reported RSRQ values: ±3.5dB Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  | Test 1: -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB Via mapping | Test 1: Noc: -86.5dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB RSRQ_52 to RSRQ_62 Test 2: Noc: -101dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB RSRQ_46 to RSRQ_60 Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: -3.5dB Ês2 / Noc: -3.5dB RSRQ_44 to RSRQ_59 |
|  | TEST CONFIGURATION 3, 6 |  |  |  |
|  | Test 1: Noc: -91dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported RSRQ values: ±2.5dB Test 2: N/A Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  | Test 1: -1.6dB 0dB 0dB Via mapping Test 2: N/A Test 3: 0dB 0dB 0dB Via mapping | Test 1: Noc: -92.6dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB RSRQ_52 to RSRQ_62 Test 2: N/A Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: -3.5dB Ês2 / Noc: -3.5dB RSRQ_44 to RSRQ_59 |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 2 SS-RSRQ from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the SS-RSRQ values are 1.5dB wider at each end for Test 1, and 0.5 dB wider at each for Tests 2 and 3. |  |  |  |  |
| 6.7.2.2.1 NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |
|  | Test 1: Noc1: -80.18 dBm/15kHz Noc2: -80.18 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB |  | Test 1: -1.5dB -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -81.68dBm/15kHz Noc2: -81.68dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 |
|  | TEST CONFIGURATION 3, 6 |  |  |  |
|  | Test 1: Noc1: -86.27 dBm/15kHz Noc2: -86.27 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB |  | Test 1: -1.53dB -1.53dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -87.8dBm/15kHz Noc2: -87.8dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 2 SS-RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the RSRQ values are 1.5dB wider at each end. |  |  |  |  |
| 6.7.2.2.2 NR SA FR1-FR1 SS-RSRQ relative measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |
|  | Test 1: Noc1: -80.18 dBm/15kHz Noc2: -80.18 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±3dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±3dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±3dB |  | Test 1: -1.5dB -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -81.68dBm/15kHz Noc2: -81.68dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB RSRQ_x-11 to RSRQ_x+2 |
|  | TEST CONFIGURATION 3, 6 |  |  |  |
|  | Test 1: Noc1: -86.27 dBm/15kHz Noc2: -86.27 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±3dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±3dB |  | Test 1: -1.53dB -1.53dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping | Test 1: Noc1: -87.8dBm/15kHz Noc2: -87.8dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB RSRQ_x-11 to RSRQ_x+2 |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 1 and Cell 2 SS-RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the RSRQ values are 1dB wider at each end. |  |  |  |  |
| 6.7.3.1 NR SA FR1 SS-SINR measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |
|  | Test 1: Noc: -93dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +4.54dB Reported SINR values: ±3.5dB Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês1 / Noc: -4dB Ês2 / Noc: -4dB Reported SINR values: ±3.5dB |  | Test 1: 0dB 0dB 0dB Via mapping Test 2: 0dB 0.5dB 0.5dB Via mapping | Test 1: Noc: -93dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +4.54dB SINR_31 to SINR_49 Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês1 / Noc: -3.5dB Ês2 / Noc: -3.5dB SINR_28 to SINR_45 |
|  | TEST CONFIGURATION 3, 6 |  |  |  |
|  | Test 1: Noc: -93dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +4.54dB Reported SINR values: ±3.5dB Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês1 / Noc: -4dB Ês2 / Noc: -4dB Reported SINR values: ±3.5dB |  | Test 1: -0.2dB 0dB 0dB Via mapping Test 2: 0dB 0.5dB 0.5dB Via mapping | Test 1: Noc: -93.2dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +4.54dB SINR_31 to SINR_49 Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês1 / Noc: -3.5dB Ês2 / Noc: -3.5dB SINR_28 to SINR_45 |
| The derivation of the SINR values takes into account the uncertainty in Cell 2 SS- SINR from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 0.5dB wider at each end. |  |  |  |  |
| 6.7.3.2.1 NR SA FR1-FR1 SS-SINR absolute measurement accuracy | TEST CONFIGURATION 1, 2, 3, 4, 5, 6 |  |  |  |
|  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported SINR values: ±3dB Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês1 / Noc1: 20dB Ês2 / Noc2: 20dB Reported SINR values: ±3dB Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -4dB Ês2 / Noc2: -4dB Reported SINR values: ±3.5dB |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0.8dB 0.8dB Via mapping | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB SINR_35 to SINR_51 Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês1 / Noc1: 20dB Ês2 / Noc2: 20dB SINR_79 to SINR_94 Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3.2dB Ês2 / Noc2: -3.2dB SINR_32 to SINR_49 |
| The derivation of the SINR values takes into account the uncertainty in Cell 2 SS- SINR from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 1dB wider at each for Tests 1 and 2 and 0.5dB wider at each end for Test 3. |  |  |  |  |
| 6.7.3.2.2 NR SA FR1-FR1 SS-SINR relative measurement accuracy | TEST CONFIGURATION 1, 2, 3, 4, 5, 6 |  |  |  |
|  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported SINR values: ±3.5dB Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês1 / Noc1: 20dB Ês2 / Noc2: 20dB Reported SINR values: ±3.5dB Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -4dB Ês2 / Noc2: -4dB Reported SINR values: ±4dB |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0.8dB 0.8dB Via mapping | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB SINR_x-10 to SINR_x+10 Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês1 / Noc1: 20dB Ês2 / Noc2: 20dB SINR_x-10 to SINR_x+10 Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3.2dB Ês2 / Noc2: -3.2dB SINR_x-11 to SINR_x+11 |
| The derivation of the SINR values takes into account the uncertainty in Cell 1 and Cell 2 SS- SINR from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 0.5dB wider at each for Tests 1 and 2 and 0dB wider at each end for Test 3. |  |  |  |  |
| 6.7.4.1.1 NR SA FR1 SSB based L1-RSRP absolute measurement accuracy |  |  |  |  |
| Test Configuration 1,2,4,5 | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±4.5dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±3dB additionally for extreme conditions) |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -94.65dBm/15kHz Ês1 / Noc1: 10dB RSRP_62 to RSRP_82 Test 2: Noc1: -119.2dBm/15kHz + ΔBG_offset Ês1 / Noc1: -2.2dB RSRP_31 to RSRP_44 RSRP_31 to RSRP_45 RSRP_32 to RSRP_45 RSRP_32 to RSRP_46 RSRP_33 to RSRP_46 RSRP_34 to RSRP_47 RSRP_34 to RSRP_48 depending on operating band |
| Test Configuration 3,6 | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±4.5dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±3dB additionally for extreme conditions) |  | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -96dBm/15kHz Ês1 / Noc1: 10dB RSRP_63 to RSRP_84 Test 2: Noc1: -116.2dBm/15kHz + ΔBG_offset Ês1 / Noc1: -2.2dB RSRP_34 to RSRP_47 RSRP_34 to RSRP_48 RSRP_35 to RSRP_48 RSRP_35 to RSRP_49 RSRP_36 to RSRP_49 RSRP_37 to RSRP_50 RSRP_37 to RSRP_51 depending on operating band |
| 6.7.4.1.2 NR SA FR1 SSB based L1-RSRP relative measurement accuracy |  |  |  |  |
| Test Configuration 1,2,4,5 | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±1dB additionally for extreme conditions) |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -94.65dBm/15kHz Ês1 / Noc1: 10dB RSRP_x-3 to RSRP_x+3 Test 2: Noc1: -119.2dBm/15kHz + ΔBG_offset Ês1 / Noc1: -2.2dB RSRP_x-3 to RSRP_x+3 |
| Test Configuration 3,6 | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±1dB additionally for extreme conditions) |  | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -96dBm/15kHz Ês1 / Noc1: 10dB RSRP_x-3 to RSRP_x+3 Test 2: Noc1: -116.2dBm/15kHz + ΔBG_offset Ês1 / Noc1: -2.2dB RSRP_x-3 to RSRP_x+3 |
| 6.7.4.2.1 NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy | Same as 6.7.4.1.1 |  | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 |
| 6.7.4.2.2 NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy | Same as 6.7.4.1.2 |  | Same as 6.7.4.1.2 | Same as 6.7.4.1.2 |
| 6.7.5.1 NR SA FR1 – E-UTRAN RSRP absolute measurement accuracy | Test 1: Noc: -91.65dBm/15kHz Ês / Noc: +10.0dB Reported RSRP values: ±9dB (±2dB additionally for extreme conditions) Test 2: Noc: -117dBm/15kHz+ ΔBG_offset Ês / Noc: -4.0dB Reported RSRP values: ±4.5dB (±3.5dB additionally for extreme conditions) |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc: -91.65dBm/15kHz Ês / Noc: 10dB RSRP_48 to RSRP_70 Test 2: Noc: -117dBm/15kHz+ ΔBG_offset Ês / Noc: -3.2dB RSRP_14 to RSRP_27 RSRP_15 to RSRP_27 RSRP_15 to RSRP_28 RSRP_16 to RSRP_28 RSRP_16 to RSRP_29 RSRP_17 to RSRP_30 RSRP_18 to RSRP_30 Depending on operating band |
| 6.7.6.1 NR SA FR1 – E-UTRAN RSRQ absolute measurement accuracy | Test 1: Noc: -83dBm/15kHz Ês / Noc: -1.75dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions) Test 2: Noc: -104.7dBm/15kHz Ês / Noc: -4.0dB Reported RSRQ values: ±3.5dB (±0.5dB additionally for extreme conditions) Test 3: Noc: -119.5dBm/15kHz+ ΔBG_offset Ês / Noc: -4.0dB Reported RSRQ values: ±3.5dB (±0.5dB additionally for extreme conditions) |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping Test 3: 0dB 0.8dB Via mapping | Test 1: Noc: -83dBm/15kHz Ês / Noc: -1.75dB RSRQ_4 to RSRQ_16 Test 2: Noc: -104.7dBm/15kHz Ês / Noc: -3.2dB RSRQ_0 to RSRQ_16 Test 3: Noc: -119.5dBm/15kHz+ ΔBG_offset Ês / Noc: -3.2dB RSRQ_0 to RSRQ_16 |
| 6.7.7.1 NR SA FR1 – E-UTRAN RS-SINR absolute measurement accuracy | Test 1: Noc: -88dBm/15kHz Ês / Noc: -1.75dB Reported RS-SINR values: ±3.0dB (±1.0dB additionally for extreme conditions) Test 2: Noc: -108.5dBm/15kHz Ês / Noc: 20.0dB Reported RS-SINR values: ±3.0dB (±1.0dB additionally for extreme conditions) Test 3: Noc: -119.5dBm/15kHz+ ΔBG_offset Ês / Noc: -4.0dB Reported RS-SINR values: ±3.5dB (±0.5dB additionally for extreme conditions) |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0dB Via mapping Test 3: 0dB 0.8dB Via mapping | Test 1: Noc: -88dBm/15kHz Ês / Noc: -1.75dB SINR_35 to SINR_51 Test 2: Noc: -108.5dBm/15kHz Ês / Noc: 20.0dB SINR_79 to SINR_94 Test 3: Noc: -119.5dBm/15kHz+ ΔBG_offset Ês / Noc: -3.2dB SINR_32 to SINR_49 |

Table F.1.3.2-4: Derivation of test requirements for EN-DC FR2 RRM tests

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 5.3.2.2.1 EN-DC FR2 contention based random access | Absolute uplink power: Pint ≥ P ≥ Pmin ±14.0dB PUMAX ≥ P > Pint ±12.0dB Relative uplink power step: Pint ≥ P ≥ Pmin ±6.0dB PUMAX ≥ P > Pint ±4.0dB Uplink timing: 120kHz SCS Te ±3.5*64*Tc | Not applicable due to UL calibration FFS dB FFS dB [48]Tc | Absolute uplink power: ±FFSdB Relative uplink power step: ±(6+FFS)dB, either PRACH being compared ≤ (Pmax – 6dB) ±(4+FFS)dB, both PRACHs being compared > (Pmax – 6dB) Uplink timing: 120kHz SCS Te ±224±[48]*Tc |
| 5.3.2.2.2 EN-DC FR2 non-contention based random access | Same as 5.3.2.2.1 | Same as 5.3.2.2.1 | Same as 5.3.2.2.1 |
| 5.4.1.1 EN-DC FR2 UE transmit timing accuracy | Test 1 (no DRX): Uplink timing: ±3*64*Tc for 240 kHz SSB SCS Max step size Tq: 2.5*64*Tc Min adjust rate Tp: 2.5*64*Tc Max adjust rate: 2.5*64*Tc Noc = -112 dBm/15 kHz Ês2 / Noc: +4.0dB Test 2 (with DRX): Uplink timing: ±3*64*Tc for 240 kHz SSB SCS Noc = -112 dBm/15 kHz Ês2 / Noc: +4.0dB | ±0.75*64*Tc +0.625*64Tc -3.725*64*Tc +1.225*64*Tc 0dB 0dB ±0.75*64*Tc 0dB 0dB | Test 1 (no DRX): Uplink timing: ±3.75*64*Tc Max step size Tq: 3.125*64*Tc Min adjust rate: -1.225*64*Tc Max adjust rate: 3.725*64*Tc Noc = -112 dBm/15 kHz Ês2 / Noc: +4.0dB Test 2 (with DRX): Uplink timing: ±3.75*64*Tc Noc = -112 dBm/15 kHz Ês2 / Noc: +4.0dB |
| 5.4.3.1 EN-DC FR2 UE timing advance adjustment accuracy | Noc = -112 dBm/15 kHz Ês1 / Noc = 4 dB UE Timing Advance Adjustment Accuracy for 120kHz SCS = ±32 Tc | 0dB 0dB +/- 40 Tc | Noc = -112 dBm/15 kHz Ês1 / Noc = 4 dB UE TAAA for 120kHz SCS = ±72 Tc |
| 5.6.1.2 EN-DC FR2 event-triggered reporting without gap in DRX | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB Signalled A3-offset: -6.0 | During T1: -3.5dB 0dB 0dB During T2: -3.5dB 0dB 0dB Signalled A3-offset: -1.0 | During T1: Noc: -101.5dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -101.5dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB Signalled A3-offset: -7.0 |
| 5.6.1.4 EN-DC FR2 event-triggered reporting with gap in DRX | Same as 5.6.1.2 | Same as 5.6.1.2 | Same as 5.6.1.2 |
| 5.6.2.1 EN-DC FR2-FR2 event-triggered reporting in non-DRX | During T1: Ês1: -87 dBm/120kHz Ês2: - infinity During T2: Ês1: -87 dBm/120kHz Ês2: -87 dBm/120kHz | During T1: 0dB 0dB During T2: 0dB 0dB | During T1: Ês1: -87 dBm/120kHz Ês2: -- infinity During T2: Ês1: -87 dBm/120kHz Ês2: -87 dBm/120kHz |
| 5.6.2.2 EN-DC FR2-FR2 event-triggered reporting in DRX | During T1: Noc: -104.70dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: -infinity During T2: Noc: -104.70dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: +9.00dB Signalled A3-offset: -6.0 | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB Signalled A3-offset: -6.0 | During T1: Noc: -104.70dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: -infinity During T2: Noc: -104.70dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: +9.00dB Signalled A3-offset: -12.0 |
| 5.6.2.3 EN-DC FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | Same as 5.6.2.1 | Same as 5.6.2.1 | Same as 5.6.2.1 |
| 5.6.2.4 EN-DC FR2-FR2 event-triggered reporting in DRX with SSB time index detection | Same as 5.6.2.2 | Same as 5.6.2.2 | Same as 5.6.2.2 |
| 5.6.2.5 EN-DC FR1-FR2 event-triggered reporting in non-DRX | During T1: Ês2: - Infinity During T2: Ês2: -87 dBm/120kHz | During T1: 0dB During T2: 0dB | During T1: Ês2: - Infinity During T2: Ês2: -87 dBm/120kHz |
| 5.6.2.6 EN-DC FR1-FR2 event-triggered reporting in DRX | During T1: Noc = -104.7 dBm/15 kHz Ês2 / Noc = - Infinity During T2: Noc = -104.7 dBm/15 kHz Ês2 / Noc = 9 dB | During T1: 0dB 0dB During T2: 0dB 0dB | During T1: Noc = -104.7 dBm/15 kHz Ês2 / Noc = - Infinity During T2: Noc = -104.7 dBm/15 kHz Ês2 / Noc = 9 dB |
| 5.6.2.7 EN-DC FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection | Same as 5.6.2.5 | Same as 5.6.2.5 | Same as 5.6.2.5 |
| 5.6.2.8 EN-DC FR1-FR2 event-triggered reporting in DRX with SSB time index detection | Same as 5.6.2.6 | Same as 5.6.2.6 | Same as 5.6.2.6 |
| 5.6.3.1 EN-DC FR2 SSB-based L1-RSRP measurement in non-DRX | During T1: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: -infinity During T2: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB | During T1: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: -infinity During T2: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB |
| 5.6.3.2 EN-DC FR2 SSB-based L1-RSRP measurement in DRX | Same as 5.6.3.1 | Same as 5.6.3.1 | Same as 5.6.3.1 |
| 5.6.3.3 EN-DC FR2 CSI-RS-based L1-RSRP measurement in non-DRX | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB Reported CSI-RSRP values ± 29dB Reported Differential CSI-RSRP values ± 7dB | 0dB 0dB 0dB Via mapping | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB CSI-RS#1: RSRP_40 to RSRP_99 CSI-RS#0: DIFFRSRP_1 to DIFFRSRP_7 |
| 5.6.3.4 EN-DC FR2 CSI-RS-based L1-RSRP measurement in DRX | Same as 5.6.3.3 | Same as 5.6.3.3 | Same as 5.6.3.3 |
| 5.7.1.1 EN-DC FR2 SS-RSRP measurement accuracy | Test 1: Noc: -91.60dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported absolute RSRP values: ±8dB Reported relative RSRP values: ±6dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 Ês2: -110.0dBm/120kHz n257 Ês3: -110.0dBm/120kHz n260 Ês2: -107.4dBm/120kHz n260 Ês3: -107.4dBm/120kHz Reported absolute RSRP values: ±8dB Reported relative RSRP values: ±6dB For extreme conditions allow ±3dB+ FFS MU additionally | Test 1: -5.80dB 0dB 0.4dB Via Mapping Test 2: 7.70dB 7.70dB 7.70dB 7.70dB Via Mapping | Test 1: Noc: -97.40dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB Cell 2: RSRP_50 to RSRP_108 Cell 3: RSRP_46 to RSRP_103 Cell 3: RSRP_x-12 to RSRP_X+2 Test 2: n257 Ês2: -102.3dBm/120kHz n257 Ês3: -102.3dBm/120kHz n260 Ês2: -99.7dBm/120kHz n260 Ês3: -99.7dBm/120kHz Band dependent. See test case tables in clause 5.7.1.1.5 |
| 5.7.1.2 EN-DC FR2-FR2 SS-RSRP measurement accuracy | Test 1 Config 1: Noc1: -90.60dBm/15kHz Noc2: -90.60dBm/15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally Test 2 Config 1: n257 Noc1: -108.13dBm/15kHz n257 Noc2: -113.13dBm/15kHz n260 Noc1: -105.53dBm/15kHz n260 Noc2: -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: -1.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally Test 1 Config 2: Noc1: -93.70dBm/15kHz Noc2: -93.70dBm/15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally Test 2 Config 2: n257 Noc1: -108.13dBm/15kHz n257 Noc2: -113.13dBm/15kHz n260 Noc1: -105.53dBm/15kHz n260 Noc2: -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: -1.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally | Test 1 Config 1: -5.70dB -5.70dB 0dB 0dB Via Mapping Via Mapping Test 2 Config 1: -6.60dB 0dB -6.60dB 0dB 0dB 2dB Via Mapping Via Mapping Test 1 Config 2: -5.60dB -5.60dB 0dB 0dB Via Mapping Via Mapping Test 2 Config 2: -6.60dB 0dB -6.60dB 0dB 0dB 2dB Via Mapping Via Mapping | Test 1 Config 1: Noc1: -96.30dBm/15kHz Noc2: -96.30dBm /15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Cell 2 n257: RSRP_41 to RSRP_109 n260: RSRP_39 to RSRP_109 Cell 3: RSRP_52 to RSRP_109 Cell 3: n257: RSRP_x-15 to RSRP_x+25 n260: RSRP_x-15 to RSRP_x+27 Test 2 Config 1: -114.73dBm/15kHz -113.13dBm/15kHz -112.13dBm/15kHz -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: 1.0dB Cell 2 n257: RSRP_33 to RSRP_101 n260: RSRP_34 to RSRP_104 Cell 3: n257: RSRP_32 to RSRP_87 n260: RSRP_34 to RSRP_90 Cell 3: n257: RSRP_x-29 to RSRP_x+11 n260: RSRP_x-29 to RSRP_x+13 Test 1 Config 2: Noc1: -99.30dBm/15kHz Noc2: -99.30dBm /15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Cell 12 n257: RSRP_41 to RSRP_109 n260: RSRP_39 to RSRP_109 Cell 3: RSRP_52 to RSRP_109 Cell 3: n257: RSRP_x-15 to RSRP_x+25 n260: RSRP_x-15 to RSRP_x+27 Test 2 Config 2: -114.73dBm/15kHz -113.13dBm/15kHz -112.13dBm/15kHz -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: 1.0dB Cell 2 n257: RSRP_36 to RSRP_104 n260: RSRP_37 to RSRP_107 Cell 3: n257: RSRP_35 to RSRP_90 n260: RSRP_37 to RSRP_93 Cell 3: n257: RSRP_x-29 to RSRP_x+11 n260: RSRP_x-29 to RSRP_x+13 |
| 5.7.2.1 EN-DC FR2 SS-RSRQ measurement accuracy | Test 1: Noc: -95.0dBm/15kHz Ês2 / Noc: +3.0dB Ês3 / Noc: +3.0dB Reported absolute RSRQ values: ±2.5dB For extreme conditions allow ±1.5dB+ FFS MU additionally Test 2: Noc: -95.0dBm/15kHz Ês2 / Noc: -3.0dB Ês3 / Noc: -3.0dB Reported absolute RSRQ values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally | Test 1: -5.70dB 0dB 0dB Via Mapping Test 2: -1.70dB 0dB 0dB Via Mapping | Test 1: Noc: -100.70dBm/15kHz Ês2 / Noc: +3.0dB Ês3 / Noc: +3.0dB Cell 3: All bands: RSRQ_41 to RSRQ_73 Test 2: Noc: -96.70dBm/15kHz Ês2 / Noc: -3.0dB Ês3 / Noc: -3.0dB Cell 3: n257, n258, n261: RSRQ_35 to RSRQ_71 n260: RSRQ_34 to RSRQ_71 |
| 5.7.2.2 EN-DC FR2-FR2 SS-RSRQ measurement accuracy | Test 1: Noc1: -94.03dBm/15kHz Noc2: -94.03dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported absolute RSRQ values: ±2.5dB For extreme conditions allow ±1.5dB+ FFS MU additionally Reported relative RSRQ values: ±3.0dB For extreme conditions allow ±1.0dB+ FFS MU additionally Test 2: Noc1: -94.03dBm/15kHz Noc2: -94.03dBm/15kHz Ês1 / Noc1: -3.0dB Ês2 / Noc2: -3.0dB Reported absolute RSRQ values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Reported relative RSRQ values: ±4.0dB For extreme conditions allow ±0.0dB+ FFS MU additionally | Test 1: -1.90dB -1.90dB 0dB 0dB Via Mapping Via Mapping Test 2: -1.41dB -1.41dB 0dB 0dB Via Mapping Via Mapping | Test 1: Noc1: -95.93dBm/15kHz Noc2: -95.93dBm /15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Cell 2 and Cell 3: RSRQ_41 to RSRQ_73 Cell 3: RSRQ_x-7 to RSRQ_x+7 Test 2: Noc1: -95.44dBm/15kHz Noc2: -95.44dBm /15kHz Ês1 / Noc1: -3.0dB Ês2 / Noc2: -3.0dB Cell 2 and Cell 3: RSRQ_37 to RSRQ_74 Cell 3: RSRQ_x-9 to RSRQ_x+9 |
| 5.7.3.1 EN-DC FR2 SS-SINR measurement accuracy | Test 1: Noc: -105.0dBm/15kHz Ês2 / Noc: +4.54dB Ês3 / Noc: +2.66dB Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Test 2: Noc: -105.0dBm/15kHz Ês2 / Noc: -3.0dB Ês3 / Noc: -3.0dB Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally | Test 1: 0dB 0dB 0dB Via Mapping Test 2: 0dB 0.2dB 0.2dB Via Mapping | Test 1: Noc: -105.0dBm/15kHz Ês2 / Noc: +4.54dB Ês3 / Noc: +2.66dB Cell 3: n257, n258, n261: SINR_22 to RSRQ_58 n260: SINR_21 to RSRQ_58 Test 2: Noc: -105.0dBm/15kHz Ês2 / Noc: -2.8dB Ês3 / Noc: -2.8dB Cell 3: n257, n258, n261: SINR_18 to RSRQ_55 n260: SINR_18 to RSRQ_54 |
| 5.7.3.2 EN-DC FR2-FR2 SS-SINR measurement accuracy | Test 1: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -0.5dB Ês2 / Noc2: -0.5dB Reported absolute SINR values: ±3.0dB For extreme conditions allow ±1.0dB+ FFS MU additionally Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Test 2: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: +11.0dB Ês2 / Noc2: +11.0dB Reported absolute SINR values: ±3.0dB For extreme conditions allow ±1.0dB+ FFS MU additionally Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Test 3: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -3.0dB Ês2 / Noc2: -3.0dB Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Reported absolute SINR values: ±4.0dB For extreme conditions allow ±0.0dB+ FFS MU additionally | Test 1: 0dB 0dB 0dB 0dB Via Mapping Via Mapping Test 2: -0.1dB -0.1dB 0dB 0dB Via Mapping Via Mapping Test 3: 0dB 0dB 0.9dB 0.9dB Via Mapping Via Mapping | Test 1: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -0.5dB Ês2 / Noc2: -0.5dB Cell 2 and Cell 3: n257, n258, n261: SINR_27 to SINR_62 n260: SINR_27 to SINR_61 Cell 3: SINR_x-8 to SINR_x+8 Test 2: Noc1: -105.1dBm/15kHz Noc2: -105.1dBm/15kHz Ês1 / Noc1: +11.0dB Ês2 / Noc2: +11.0dB Cell 2 and Cell 3: n257, n258, n261: SINR_48 to SINR_87 n260: SINR_48 to SINR_86 Cell 3: SINR_x-17 to SINR_x+17 Test 3: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -2.1dB Ês2 / Noc2: -2.1dB Cell 2 and Cell 3: n257, n258, n261: SINR_23 to SINR_60 n260: SINR_22 to SINR_59 Cell 3: SINR_x-9 to SINR_x+9 |

Table F.1.3.2-5: Derivation of test requirements for NR SA FR2 RRM tests

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 7.3.2.2.1 NR SA FR2 contention based random access | Absolute uplink power: Pint ≥ P ≥ Pmin ±14.0dB PUMAX ≥ P > Pint ±12.0dB Relative uplink power step: Pint ≥ P ≥ Pmin ±6.0dB PUMAX ≥ P > Pint ±4.0dB Uplink timing: 120kHz SCS Te ±3.5*64*Tc | Not applicable due to UL calibration FFS dB FFS dB [48]Tc | Absolute uplink power: ±FFSdB Relative uplink power step: ±(6+FFS)dB, either PRACH being compared ≤ (Pmax – 6dB) ±(4+FFS)dB, both PRACHs being compared > (Pmax – 6dB) Uplink timing: 120kHz SCS Te ±224±[48]*Tc |
| 7.3.2.2.2 NR SA FR2 non-contention based random access | Same as 7.3.2.2.1 | Same as 7.3.2.2.1 | Same as 7.3.2.2.1 |
| 7.4.1.1 SA FR2 UE transmit timing accuracy | Test 1 (no DRX): Uplink timing: ±3*64*Tc for 240 kHz SSB SCS Max step size Tq: 2.5*64*Tc Min adjust rate Tp: 2.5*64*Tc Max adjust rate: 2.5*64*Tc Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB Test 2 (with DRX): Uplink timing: ±3*64*Tc for 240 kHz SSB SCS Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB | ±0.75*64*Tc +0.625*64Tc -3.725*64*Tc +1.225*64*Tc 0dB 0dB ±0.75*64*Tc 0dB 0dB | Test 1 (no DRX): Uplink timing: ±3.75*64*Tc Max step size Tq: 3.125*64*Tc Min adjust rate: -1.225*64*Tc Max adjust rate: 3.725*64*Tc Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB Test 2 (with DRX): Uplink timing: ±3.75*64*Tc Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB |
| 7.4.3.1 NR SA FR2 UE timing advance adjustment accuracy | Noc = -112 dBm/15 kHz Ês1 / Noc = 4 dB UE Timing Advance Adjustment Accuracy for 120kHz SCS = ±32 Tc | 0dB 0dB +/- 40 Tc | Noc = -112 dBm/15 kHz Ês1 / Noc = 4 dB UE TAAA for 120kHz SCS = ±72 Tc |
| 7.6.2.1 NR SA FR2-FR2 event-triggered reporting in non-DRX | During T1: Ês1: -87 dBm/120kHz Ês2: - infinity During T2: Ês1: -87 dBm/120kHz Ês2: -87 dBm/120kHz | During T1: 0dB 0dB During T2: 0dB 0dB | During T1: Ês1: -87 dBm/120kHz Ês2: - infinity During T2: Ês1: -87 dBm/120kHz Ês2: -87 dBm/120kHz |
| 7.6.2.3 NR SA FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | Same as 7.6.2.1 | Same as 7.6.2.1 | Same as 7.6.2.1 |
| 7.6.2.5 SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1) | During T1: Ês2: - Infinity During T2: Ês2: -87 dBm/120kHz | During T1: 0dB During T2: 0dB | During T1: Ês2: - Infinity During T2: Ês2: -87 dBm/120kHz |
| 7.6.2.6 SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (PCell in FR1) | During T1: Noc = -104.7 dBm/15 kHz Ês2 / Noc = - Infinity During T2: Noc = -104.7 dBm/15 kHz Ês2 / Noc = 9 dB | During T1: 0dB 0dB During T2: 0dB 0dB | During T1: Noc = -104.7 dBm/15 kHz Ês2 / Noc = - Infinity During T2: Noc = -104.7 dBm/15 kHz Ês2 / Noc = 9 dB |
| 7.6.2.7 SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1) | Same as 5.6.2.5 | Same as 5.6.2.5 | Same as 5.6.2.5 |
| 7.6.2.8 SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1) | Same as 5.6.2.6 | Same as 5.6.2.6 | Same as 5.6.2.6 |
| 7.6.3.1 NR SA FR2 SSB-based L1-RSRP measurement in non-DRX | Same as 5.6.3.1 | Same as 5.6.3.1 | Same as 5.6.3.1 |
| 7.6.3.2 NR SA FR2 SSB-based L1-RSRP measurement in DRX | Same as 5.6.3.1 | Same as 5.6.3.1 | Same as 5.6.3.1 |
| 7.6.3.3 NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB Reported CSI-RSRP values ± 29dB Reported Differential CSI-RSRP values ± 7dB | 0dB 0dB 0dB Via mapping | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB CSI-RS#1: RSRP_40 to RSRP_99 CSI-RS#0: DIFFRSRP_1 to DIFFRSRP_7 |
| 7.6.3.4 NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX | Same as 7.6.3.3 | Same as 7.6.3.3 | Same as 7.6.3.3 |
| 7.7.1.1 NR SA FR2 SS-RSRP measurement accuracy | Test 1: Noc: -91.60dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported absolute RSRP values: ±8dB Reported relative RSRP values: ±6dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 Ês1: -110.0dBm/120kHz n257 Ês2: -110.0dBm/120kHz n260 Ês1: -107.4dBm/120kHz n260 Ês2: -107.4dBm/120kHz Reported absolute RSRP values: ±8dB Reported relative RSRP values: ±6dB For extreme conditions allow ±3dB+ FFS MU additionally | Test 1: -5.80dB 0dB 0.4dB Via Mapping Test 2: 7.70dB 7.70dB 7.70dB 7.70dB Via Mapping | Test 1: Noc: -97.40dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB Cell 1: RSRP_50 to RSRP_108 Cell 2: RSRP_46 to RSRP_103 Cell 2: RSRP_x-12 to RSRP_X+2 Test 2: n257 Ês1: -102.3dBm/120kHz n257 Ês2: -102.3dBm/120kHz n260 Ês1: -99.7dBm/120kHz n260 Ês2: -99.7dBm/120kHz Band dependent. See test case tables in clause 5.7.1.1.5 |
| 7.7.1.2 NR SA FR2-FR2 SS-RSRP measurement accuracy | Test 1 Config 1: Noc1: -90.60dBm/15kHz Noc2: -90.60dBm/15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally Test 2 Config 1: n257 Noc1: -108.13dBm/15kHz n257 Noc2: -113.13dBm/15kHz n260 Noc1: -105.53dBm/15kHz n260 Noc2: -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: -1.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally Test 1 Config 2: Noc1: -93.70dBm/15kHz Noc2: -93.70dBm/15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally Test 2 Config 2: n257 Noc1: -108.13dBm/15kHz n257 Noc2: -113.13dBm/15kHz n260 Noc1: -105.53dBm/15kHz n260 Noc2: -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: -1.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally | Test 1 Config 1: -5.70dB -5.70dB 0dB 0dB Via Mapping Via Mapping Test 2 Config 1: -6.60dB 0dB -6.60dB 0dB 0dB 2dB Via Mapping Via Mapping Test 1 Config 2: -5.60dB -5.60dB 0dB 0dB Via Mapping Via Mapping Test 2 Config 2: -6.60dB 0dB -6.60dB 0dB 0dB 2dB Via Mapping Via Mapping | Test 1 Config 1: Noc1: -96.30dBm/15kHz Noc2: -96.30dBm /15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Cell 1 n257: RSRP_41 to RSRP_109 n260: RSRP_39 to RSRP_109 Cell 2: RSRP_52 to RSRP_109 Cell 2: n257: RSRP_x-15 to RSRP_x+25 n260: RSRP_x-15 to RSRP_x+27 Test 2 Config 1: -114.73dBm/15kHz -113.13dBm/15kHz -112.13dBm/15kHz -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: 1.0dB Cell 1 n257: RSRP_33 to RSRP_101 n260: RSRP_34 to RSRP_104 Cell 2: n257: RSRP_32 to RSRP_87 n260: RSRP_34 to RSRP_90 Cell 2: n257: RSRP_x-29 to RSRP_x+11 n260: RSRP_x-29 to RSRP_x+13 Test 1 Config 2: Noc1: -99.30dBm/15kHz Noc2: -99.30dBm /15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Cell 1 n257: RSRP_41 to RSRP_109 n260: RSRP_39 to RSRP_109 Cell 2: RSRP_52 to RSRP_109 Cell 2: n257: RSRP_x-15 to RSRP_x+25 n260: RSRP_x-15 to RSRP_x+27 Test 2 Config 2: -114.73dBm/15kHz -113.13dBm/15kHz -112.13dBm/15kHz -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: 1.0dB Cell 1 n257: RSRP_36 to RSRP_104 n260: RSRP_37 to RSRP_107 Cell 2: n257: RSRP_35 to RSRP_90 n260: RSRP_37 to RSRP_93 Cell 2: n257: RSRP_x-29 to RSRP_x+11 n260: RSRP_x-29 to RSRP_x+13 |
| 7.7.2.1 NR SA FR2 SS-RSRQ measurement accuracy | Test 1: Noc: -95.0dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported absolute RSRQ values: ±2.5dB For extreme conditions allow ±1.5dB+ FFS MU additionally Test 2: Noc: -95.0dBm/15kHz Ês1 / Noc: -3.0dB Ês2 / Noc: -3.0dB Reported absolute RSRQ values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally | Test 1: -5.70dB 0dB 0dB Via Mapping Test 2: -1.70dB 0dB 0dB Via Mapping | Test 1: Noc: -100.70dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Cell 2: All bands: RSRQ_41 to RSRQ_73 Test 2: Noc: -96.70dBm/15kHz Ês1 / Noc: -3.0dB Ês2 / Noc: -3.0dB Cell 2: n257, n258, n261: RSRQ_35 to RSRQ_71 n260: RSRQ_34 to RSRQ_71 |
| 7.7.2.2 NR SA FR2-FR2 SS-RSRQ measurement accuracy | Test 1: Noc1: -94.03dBm/15kHz Noc2: -94.03dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported absolute RSRQ values: ±2.5dB For extreme conditions allow ±1.5dB+ FFS MU additionally Reported relative RSRQ values: ±3.0dB For extreme conditions allow ±1.0dB+ FFS MU additionally Test 2: Noc1: -94.03dBm/15kHz Noc2: -94.03dBm/15kHz Ês1 / Noc1: -3.0dB Ês2 / Noc2: -3.0dB Reported absolute RSRQ values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Reported relative RSRQ values: ±4.0dB For extreme conditions allow ±0.0dB+ FFS MU additionally | Test 1: -1.90dB -1.90dB 0dB 0dB Via Mapping Via Mapping Test 2: -1.41dB -1.41dB 0dB 0dB Via Mapping Via Mapping | Test 1: Noc1: -95.93dBm/15kHz Noc2: -95.93dBm /15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Cell 1 and Cell 2: RSRQ_41 to RSRQ_73 Cell 2: RSRQ_x-7 to RSRQ_x+7 Test 2: Noc1: -95.44dBm/15kHz Noc2: -95.44dBm /15kHz Ês1 / Noc1: -3.0dB Ês2 / Noc2: -3.0dB Cell 1 and Cell 2: RSRQ_37 to RSRQ_74 Cell 2: RSRQ_x-9 to RSRQ_x+9 |
| 7.7.3.1 NR SA FR2 SS-SINR measurement accuracy | Test 1: Noc: -105.0dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +2.66dB Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Test 2: Noc: -105.0dBm/15kHz Ês1 / Noc: -3.0dB Ês2 / Noc: -3.0dB Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally | Test 1: 0dB 0dB 0dB Via Mapping Test 2: 0dB 0.2dB 0.2dB Via Mapping | Test 1: Noc: -105.0dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +2.66dB Cell 2: n257, n258, n261: SINR_22 to RSRQ_58 n260: SINR_21 to RSRQ_58 Test 2: Noc: -105.0dBm/15kHz Ês1 / Noc: -2.8dB Ês2 / Noc: -2.8dB Cell 2: n257, n258, n261: SINR_18 to RSRQ_55 n260: SINR_18 to RSRQ_54 |
| 7.7.3.2 NR SA FR2-FR2 SS-SINR measurement accuracy | Test 1: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -0.5dB Ês2 / Noc2: -0.5dB Reported absolute SINR values: ±3.0dB For extreme conditions allow ±1.0dB+ FFS MU additionally Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Test 2: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: +11.0dB Ês2 / Noc2: +11.0dB Reported absolute SINR values: ±3.0dB For extreme conditions allow ±1.0dB+ FFS MU additionally Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Test 3: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -3.0dB Ês2 / Noc2: -3.0dB Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Reported absolute SINR values: ±4.0dB For extreme conditions allow ±0.0dB+ FFS MU additionally | Test 1: 0dB 0dB 0dB 0dB Via Mapping Via Mapping Test 2: -0.1dB -0.1dB 0dB 0dB Via Mapping Via Mapping Test 3: 0dB 0dB 0.9dB 0.9dB Via Mapping Via Mapping | Test 1: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -0.5dB Ês2 / Noc2: -0.5dB Cell 1 and Cell 2: n257, n258, n261: SINR_27 to SINR_62 n260: SINR_27 to SINR_61 Cell 2: SINR_x-8 to SINR_x+8 Test 2: Noc1: -105.1dBm/15kHz Noc2: -105.1dBm/15kHz Ês1 / Noc1: +11.0dB Ês2 / Noc2: +11.0dB Cell 1 and Cell 2: n257, n258, n261: SINR_48 to SINR_87 n260: SINR_48 to SINR_86 Cell 2: SINR_x-17 to SINR_x+17 Test 3: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -2.1dB Ês2 / Noc2: -2.1dB Cell 1 and Cell 2: n257, n258, n261: SINR_23 to SINR_60 n260: SINR_22 to SINR_59 Cell 2: SINR_x-9 to SINR_x+9 |

Table F.1.3.2-6: Derivation of test requirements for E-UTRA – NR inter-RAT test cases with E-UTRA serving cell RRM tests

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 8.2.1.1 E-UTRA – NR FR1 cell re-selection to higher priority NR target cell | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: +12dB | During T1: 0dB -1.90dB 0dB 0.35dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 1.55dB 1.55dB | During T1: Noc1: -98dBm/15kHz Noc2: -99.90dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -3.65dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.55dB Ês2 / Noc2: +13.55dB |
| 8.2.1.2 E-UTRA – NR FR1 Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16 | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: 14dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +12dB Ês2 / Noc2: +14dB | During T1: -1.90dB -1.90dB 0.35dB 3.45dB During T2: -1.90dB -1.90dB 3.45dB 0dB | During T1: Noc1: -99.9dBm/15kHz Noc2: -99.9dBm/15kHz Ês1 / Noc1: -3.65dB Ês2 / Noc2: 17.45dB During T2: Noc1: -99.9dBm/15kHz Noc2: -99.9dBm/15kHz Ês1 / Noc1: +15.45dB Ês2 / Noc2: +14dB |
| 8.3.1.1 E-UTRA – NR FR1 handover with known target cell | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: 0dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: 0dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB -1.55dB +1.55dB During T3: 0dB 0dB -1.55dB +1.55dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5.45dB Ês2 / Noc2: +1.55dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5.45dB Ês2 / Noc2: +1.55dB |
| 8.4.1.1 E-UTRA – NR FR1 SFTD measurement delay in non-DRX | During T1: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +4dB | During T1: 0dB 0dB 0dB 0dB | During T1: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +4dB |
| 8.4.1.2 E-UTRA – NR FR1 SFTD measurement delay in DRX | Same as 8.4.1.1 | Same as 8.4.1.1 | Same as 8.4.1.1 |
| 8.4.2.1 E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in non-DRX | During T1: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: -infinity During T2: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +7dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB -1.65dB +1.65dB | During T1: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.35dB Ês2 / Noc2: +8.65dB |
| 8.4.2.2 E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in DRX | Same as 8.4.2.1 | Same as 8.4.2.1 | Same as 8.4.2.1 |
| 8.4.2.3 E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in non-DRX | Same as 8.4.2.1 | Same as 8.4.2.1 | Same as 8.4.2.1 |
| 8.4.2.4 E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in DRX | Same as 8.4.2.1 | Same as 8.4.2.1 | Same as 8.4.2.1 |
| 8.4.2.9 E-UTRA – NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16 | During T1: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: -infinity During T2: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +7dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: -infinity During T2: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +7dB |
| 8.5.1.1 E-UTRA – NR FR1 SFTD measurement accuracy | Test 1 Ês1 / Noc1: -3dB Ês2 / Noc2: -3dB | Test 1: +0.8dB +0.3dB | Test 1: Ês1 / Noc1: -2.2dB Ês2 / Noc2: -2.7dB |
| 8.5.2.1.1.1 E-UTRA SS-RSRP absolute measurement accuracy of a NR FR1 neighbour cell |  |  |  |
| Test Configuration 1,2,4,5 | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: +10.0dB Reported RSRP values: ±8dB (±3.0dB additionally for extreme conditions) Test 2: Noc1: -117dBm/15kHz + ΔBG_offset Ês1 / Noc1: -4.0dB Reported RSRP values: ±4.5dB (±4.5dB additionally for extreme conditions) | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -94.65dBm/15kHz Ês1 / Noc1: +10.0dB RSRP_62 to RSRP_82 Test 2: Noc1: -117dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3.2dB RSRP_30 to RSRP_43 RSRP_31 to RSRP_43 RSRP_31 to RSRP_44 RSRP_32 to RSRP_44 RSRP_32 to RSRP_45 RSRP_33 to RSRP_45 RSRP_33 to RSRP_46 RSRP_34 to RSRP_46 depending on operating band |
| Test Configuration 3,6 | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: +10.0dB Reported RSRP values: ±8dB (±3.0dB additionally for extreme conditions) Test 2: Noc1: -117dBm/15kHz + ΔBG_offset Ês1 / Noc1: -4.0dB Reported RSRP values: ±4.5dB (±4.5dB additionally for extreme conditions) | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -96.00dBm/15kHz Ês1 / Noc1: +10.0dB RSRP_64 to RSRP_83 Test 2: Noc1: -117dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3.2dB RSRP_33 to RSRP_46 RSRP_34 to RSRP_46 RSRP_34 to RSRP_47 RSRP_35 to RSRP_47 RSRP_35 to RSRP_48 RSRP_36 to RSRP_48 RSRP_36 to RSRP_49 RSRP_37 to RSRP_49 depending on operating band |
| 8.5.2.1.2 E-UTRA SS-RSRP absolute measurement accuracy of a NR FR2 neighbour cell | Test 1: Noc: -105dBm/15kHz Ês / Noc 11dB Reported RSRP values: ±8dB (±3dB additionally for extreme conditions) Test 2: Ês FR2a PC3: -109.1dBm/SCS Reported RSRP values: ±8dB Ês FR2b PC3: -106.5dBm/SCS Reported RSRP values: ±8dB Ês FR2c PC3: -105.5dBm/SCS Reported RSRP values: ±8dB (±3dB additionally for extreme conditions in all frequency ranges) | Test 1: -0.1dB 0dB Via mapping Test 2: +5.65dB Via mapping +5.65dB Via mapping +5.65dB Via mapping | Test 1: Noc: -105.1dBm/15kHz Ês / Noc: +11.0dB RSRP_48 to RSRP_105 Test 2: Ês: -103.45dBm/SCS RSRP_29 to RSRP_87 Ês: -100.85dBm/SCS RSRP_32 to RSRP_89 Ês: -99.85dBm/SCS RSRP_33 to RSRP_90 |
| 8.5.2.2.1 E-UTRA SS-RSRQ absolute measurement accuracy of a NR FR1 neighbour cell |  |  |  |
| Test Configuration 1,2,4,5 | Test 1: Noc1: -80.18 dBm/15kHz Ês1 / Noc1: -1.75dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions) Test 2: Noc1: -106dBm/15kHz Ês1 / Noc1: -1.75dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions) Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: -1.75dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions) | Test 1: -1.5dB 0dB Via mapping Test 2: 0dB 0dB Via mapping Test 3: 0dB 0dB Via mapping | Test 1: Noc1: -81.68dBm/15kHz Ês1 / Noc1: -1.75dB RSRQ_51 to RSRQ_63 Test 2: Noc1: -106dBm/15kHz Ês1 / Noc1: -1.75dB RSRQ_51 to RSRQ_63 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: -1.75dB RSRQ_51 to RSRQ_63 |
| Test Configuration 3,6 | Test 1: Noc1: -86.27 dBm/15kHz Ês1 / Noc1: -1.75dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions) Test 2: Noc1: -113dBm/15kHz Ês1 / Noc1: -1.75dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions) Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: -1.75dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions) | Test 1: -1.53dB 0dB Via mapping Test 2: 0dB 0dB Via mapping Test 3: 0dB 0dB Via mapping | Test 1: Noc1: -87.80dBm/15kHz Ês1 / Noc1: -1.75dB RSRQ_51 to RSRQ_63 Test 2: Noc1: -113dBm/15kHz Ês1 / Noc1: -1.75dB RSRQ_51 to RSRQ_63 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: -1.75dB RSRQ_51 to RSRQ_63 |
| 8.5.2.2.2 E-UTRA SS-RSRQ absolute measurement accuracy of a NR FR2 neighbour cell | Test 1: Noc1: -105dBm/15kHz Ês1 / Noc1: -0.5dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions) Test 2: Noc1 FR2a PC3: -115.1dBm/15kHz Noc1 FR2b PC3: -112.5dBm/15kHz Noc1 FR2c PC3: -111.5dBm/15kHz Ês1 / Noc1: -1.75dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions in all frequency ranges) | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 5.3dB Via mapping | Test 1: Noc1: -105dBm/15kHz Ês1 / Noc1: -0.5dB RSRQ_23 to RSRQ_95 Test 2: Noc1: -115.1dBm/15kHz Noc1: -112.5dBm/15kHz Noc1: -111.5dBm/SCS Ês1 / Noc1: 3.55dB RSRQ_26 to RSRQ_98 |
| 8.5.2.3.1 E-UTRA SS-SINR absolute measurement accuracy of a NR FR1 neighbour cell | Test 1: Noc1: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Reported SINR values: ±3dB Test 2: Noc1: -108.5dBm/15kHz Ês1 / Noc1: 20dB Reported SINR values: ±3dB Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -4dB Reported SINR values: ±3.5dB | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0dB Via mapping Test 3: 0dB 0.8dB Via mapping | Test 1: Noc1: -88 dBm/15kHz Ês1 / Noc1: -1.75dB SINR_35 to SINR_51 Test 2: Noc1: -108.5dBm/15kHz Ês1 / Noc1: 20dB SINR_79 to SINR_94 Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3.2dB SINR_32 to SINR_49 |
| 8.5.2.3.2 E-UTRA SS-SINR absolute measurement accuracy of a NR FR2 neighbour cell | Test 1: Noc1: -105dBm/15kHz Ês1 / Noc1: -0.5dB Reported SINR values: ±3dB (±1dB additionally for extreme conditions) Test 2: Noc1: -105dBm/15kHz Ês1 / Noc1: 11dB Reported SINR values: ±3dB (±1dB additionally for extreme conditions) Test 3: Noc1 FR2a PC3: -115.1dBm/15kHz Noc1 FR2b PC3: -112.5dBm/15kHz Noc1 FR2c PC3: -111.5dBm/15kHz Ês1 / Noc1: -1dB Reported SINR values: ±3dB (±1dB additionally for extreme conditions in all frequency ranges) | Test 1: 0dB 0dB Via mapping Test 2: -0.1dB 0dB Via mapping Test 3: 0dB 0dB 0dB 4.6dB Via mapping | Test 1: Noc1: -105dBm/15kHz Ês1 / Noc1: -0.5dB SINR_9 to SINR_82 Test 2: Noc1: -105.1dBm/15kHz Ês1 / Noc1: 11dB SINR_32 to SINR_105 Test 3: Noc1: -115.1dBm/15kHz Noc1: -112.5dBm/15kHz Noc1: -111.5dBm/SCS Ês1 / Noc1: 3.6dB SINR_17 to SINR_90 |

Annex G (normative):
Statistical testing

# G.1	General

The test requirements are expressed as absolute requirements with a single value stating the requirement or expressed as a success rate. The statistical nature depends on the type of test requirement. Some have large statistical variations, while others are not statistical in nature at all. When testing a parameter with a statistical nature, a confidence level is set. This establishes the probability that a Device Under Test (DUT) passing the test actually meets the test requirement and determines how many times a test have to be repeated and what the pass and fail criteria is. This Annex describes how to set the statistical significance.

# G.2	Statistical testing of delay and UE measurement performance in RRM tests

## G.2.1	General

The RRM tests are either of deterministic or of statistical nature. The pass fail limits in tests of statistical nature are expressed as a limit (e.g. delay limit) and a success ratio applicable for the limit. The success ratio is 90% uniform (the complement is the error ratio ER = 10%).

## G.2.2	Design of the test

The test is defined by the following design principles (see TS 36.521-1 clause G.X, Theory):

1) The early decision concept is applied.

2) A second limit is introduced: bad DUT factor M>1

To decide the test pass:

Supplier risk is applied based on the bad DUT quality

To decide the test fails

Customer risk is applied based on the specified DUT quality

The test is defined by the following parameters:

1) Limit ER = 0.1 (success ratio = 90%)

2) Bad DUT factor M=1.5 (selectivity)

3) Confidence level CL = 95% (for specified DUT and bad DUT-quality)

## G.2.3	Numerical definition of the pass fail limits

Editor’s Note:

-	Further investigate the technical details behind this statistical method to ensure that this is applicable for FR2 radiated test cases.

Table G.2.3-1: pass fail limits

| ne | nsp | nsf | ne | nsp | nsf | ne | nsp | nsf | ne | nsp | nsf |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 33 | NA | 43 | 408 | 283 | 86 | 737 | 644 | 129 | 1056 | 1021 |
| 1 | 46 | NA | 44 | 416 | 291 | 87 | 745 | 653 | 130 | 1064 | 1030 |
| 2 | 58 | NA | 45 | 424 | 299 | 88 | 752 | 661 | 131 | 1071 | 1039 |
| 3 | 69 | NA | 46 | 432 | 307 | 89 | 760 | 670 | 132 | 1078 | 1048 |
| 4 | 79 | NA | 47 | 440 | 315 | 90 | 767 | 679 | 133 | 1086 | 1057 |
| 5 | 89 | NA | 48 | 447 | 324 | 91 | 775 | 687 | 134 | 1093 | 1066 |
| 6 | 99 | NA | 49 | 455 | 332 | 92 | 782 | 696 | 135 | 1100 | 1074 |
| 7 | 109 | NA | 50 | 463 | 340 | 93 | 790 | 705 | 136 | 1108 | 1083 |
| 8 | 118 | NA | 51 | 471 | 348 | 94 | 797 | 713 | 137 | 1115 | 1092 |
| 9 | 127 | NA | 52 | 478 | 356 | 95 | 804 | 722 | 138 | 1122 | 1101 |
| 10 | 136 | 39 | 53 | 486 | 365 | 96 | 812 | 731 | 139 | 1130 | 1110 |
| 11 | 145 | 45 | 54 | 494 | 373 | 97 | 819 | 739 | 140 | 1137 | 1119 |
| 12 | 154 | 51 | 55 | 502 | 381 | 98 | 827 | 748 | 141 | 1144 | 1128 |
| 13 | 163 | 58 | 56 | 509 | 389 | 99 | 834 | 757 | 142 | 1152 | 1137 |
| 14 | 172 | 64 | 57 | 517 | 398 | 100 | 842 | 766 | 143 | 1159 | 1147 |
| 15 | 180 | 71 | 58 | 525 | 406 | 101 | 849 | 774 | 144 | 1166 | 1155 |
| 16 | 189 | 78 | 59 | 532 | 414 | 102 | 857 | 783 | 145 | 1174 | 1164 |
| 17 | 197 | 85 | 60 | 540 | 423 | 103 | 864 | 792 | 146 | 1181 | 1173 |
| 18 | 206 | 92 | 61 | 548 | 431 | 104 | 871 | 801 | 147 | NA | 1182 |
| 19 | 214 | 99 | 62 | 555 | 440 | 105 | 879 | 809 | 148 |  |  |
| 20 | 223 | 106 | 63 | 563 | 448 | 106 | 886 | 818 | 149 |  |  |
| 21 | 231 | 113 | 64 | 571 | 456 | 107 | 894 | 827 | 150 |  |  |
| 22 | 239 | 120 | 65 | 578 | 465 | 108 | 901 | 836 | 151 |  |  |
| 23 | 248 | 128 | 66 | 586 | 473 | 109 | 909 | 844 | 152 |  |  |
| 24 | 256 | 135 | 67 | 594 | 482 | 110 | 916 | 853 | 153 |  |  |
| 25 | 264 | 142 | 68 | 601 | 490 | 111 | 923 | 862 | 154 |  |  |
| 26 | 272 | 150 | 69 | 609 | 499 | 112 | 931 | 871 | 155 |  |  |
| 27 | 281 | 157 | 70 | 616 | 507 | 113 | 938 | 880 | 156 |  |  |
| 28 | 289 | 165 | 71 | 624 | 516 | 114 | 946 | 888 | 157 |  |  |
| 29 | 297 | 173 | 72 | 632 | 524 | 115 | 953 | 897 | 158 |  |  |
| 30 | 305 | 180 | 73 | 639 | 533 | 116 | 960 | 906 | 159 |  |  |
| 31 | 313 | 188 | 74 | 647 | 541 | 117 | 968 | 915 | 160 |  |  |
| 32 | 321 | 196 | 75 | 654 | 550 | 118 | 975 | 924 | 161 |  |  |
| 33 | 329 | 204 | 76 | 662 | 558 | 119 | 983 | 933 | 162 |  |  |
| 34 | 337 | 211 | 77 | 669 | 567 | 120 | 990 | 941 | 163 |  |  |
| 35 | 345 | 219 | 78 | 677 | 575 | 121 | 997 | 950 | 164 |  |  |
| 36 | 353 | 227 | 79 | 684 | 584 | 122 | 1005 | 959 | 165 |  |  |
| 37 | 361 | 235 | 80 | 692 | 592 | 123 | 1012 | 968 | 166 |  |  |
| 38 | 369 | 243 | 81 | 700 | 601 | 124 | 1019 | 977 | 167 |  |  |
| 39 | 377 | 251 | 82 | 707 | 610 | 125 | 1027 | 986 | 168 |  |  |
| 40 | 385 | 259 | 83 | 715 | 618 | 126 | 1034 | 994 | 169 |  |  |
| 41 | 393 | 267 | 84 | 722 | 627 | 127 | 1042 | 1003 |  |  |  |
| 42 | 400 | 275 | 85 | 730 | 635 | 128 | 1049 | 1012 |  |  |  |

The first column is the number of errors (ne = number of exceeded delays or number of wrong reports)

The second column is the number of samples for the pass limit (nsp , ns=Number of samples= number of successes + number of exceedings or number of reports)

The third column is the number of samples for the fail limit (nsf)

## G.2.4	Pass fail decision rules

The pass fail decision rules apply for a single test, comprising one component in the test vector. The over all Pass /Fail conditions are defined in clause G.2.6

Having observed 0 errors, pass the test at 33+ samples, otherwise continue

Having observed 1 error, pass the test at 46+ samples, otherwise continue

Having observed 2 errors, pass the test at 58+ samples, otherwise continue

Having observed 10 errors, pass the test at 136+ samples, fail the test at 39 samples, otherwise continue

Having observed 146 errors, pass the test at 1181+ samples, fail the test at 1173- samples, otherwise continue

Having observed 147 errors, fail the test at 1182- samples,

Where x+ means: x or more, x- means x or less

NOTE 1: an ideal DUT passes after 33 samples. The maximum test time is 1181 samples.

## G.2.5	Void

## G.2.6	Test conditions for delay tests and UE measurement performance

Table G.2.6-1: test conditions

| Test | Statistical independence | Number of components in the test vector, as specified in the test requirements and initial conditions of the applicable test | Over all Pass/Fail condition |
| --- | --- | --- | --- |
| All tests in clauses 4.4.3, 4.5, 4.6, 5.4.3, 5.5, 5.6, 6.1, 6.2, 6.3.1, 6.3.2.1, 6.3.2.3, 6.4.3, 6.5, 6.6, 7.1, 7.2, 7.3.1, 7.3.2.1, 7.3.2.3, 7.4.3, 7.5, 7.6 are delay tests of statistical nature while 4.3.2.2, 4.4.1, 5.3.2.2, 5.4.1, 6.3.2.2, 6.4.1, 7.3.2.2, 7.4.1 are not applicable, since they are deterministic. | Test procedure in all statistical tests ensures independency | 1 per operating band (if tested, see 3A.3) | Full set of environmental conditions (5) per operating band |
| All tests in clauses 4.7, 5.7, 6.7 and 7.7 are UE level reports of statistical nature | Independency is assumed, although Layer 1 filtering is applied to the reported results | Full set of environmental conditions (5) per operating band | Full set of environmental conditions (5) per operating band |

# G.X	Theory to derive the numbers in Table G.2.3-1 (informative)

TS 36.521-1 Annex G.X applies.

Annex H (normative):
Default message contents for RRM

# H.1	Void

# H.2	System information blocks message content exceptions

## H.2.1	System information blocks message contents exceptions for NR intra frequency cell re-selection

SIB2: for NR intra-frequency cell re-selection

Table H.2.1-1: SIB2: NR intra frequency cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionInfoCommon SEQUENCE { |  |  |  |
| rangeToBestCell | Not present |  |  |
| } |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -70 | -140 is actual value in dBm (-70 * 2 dBm) | SCS15 |
|  | -69 | -138 is actual value in dBm (-69 * 2 dBm) | SCS30 |
|  | -69 | -138 is actual value in dBm (-69 * 2 dBm) | SCS120 |
|  | -68 | -136 is actual value in dBm (-68 * 2 dBm) | SCS240 |
| s-IntraSearchP | 30 | 60 is actual value in dB (30 * 2 dB) |  |
| smtc | SSB-MTC specified in TS 38.508-1 [14] Table 7.3.1-3 with condition SMTC.n |  | SMTC.n |
| deriveSSB-IndexFromCell | false |  | Asynchronous cells |
|  | true |  | Synchronous cells |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SMTC.n | SMTC pattern n according to TS 38.133 [6] A.3.11 |
| Synchronous cells | SSB indices of neigibour cells can be derived from timing of serving cell |
| Asynchronous cells | SSB indices of neigibour cells can not be derived from timing of serving cell |

SIB3: for NR intra-frequency cell re-selection

Table H.2.1-2: SIB3: NR intra frequency cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB3 ::= SEQUENCE { |  |  |  |
| intraFreqNeighCellList SEQUENCE (SIZE (1..maxCellIntra)) OF SEQUENCE { |  |  |  |
| IntraFreqNeighCellInfo ::= SEQUENCE{ |  |  |  |
| physCellId | Set according to the neighbour cell PCI |  |  |
| q-OffsetCell | dB0 | 0 is actual value in dB (0 * 2 dB) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

SIB1: for NR intra frequency cell re-selection

Table H.2.1-3: SIB1: NR intra frequency cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |  |
| q-RxLevMin |  | -70 | -140 is actual value in dBm (-70 * 2 dBm) | SCS15 |
|  |  | -69 | -138 is actual value in dBm (-69 * 2 +1 dBm) | SCS30 |
|  |  | -69 | -138 is actual value in dBm (-69 * 2 dBm) | SCS120 |
|  |  | -68 | -136 is actual value in dBm (-68 * 2 dBm) | SCS240 |
| } |  |  |  |  |
| servingCellConfigCommon SEQUENCE { |  |  |  |  |
| highSpeedConfig-r16 SEQUENCE { |  |  |  |  |
| highSpeedMeasFlag-r16 |  | true |  | HighSpeedMeas |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| HighSpeedMeas | highSpeedMeasFlag-r16 is configured. UE shall apply the enhanced measurement requirements to support high speed up to 500 km/h as specified in TS 38.133 [14] |

## H.2.2	System information blocks message contents exceptions for NR inter frequency cell re-selection

SIB2: for NR inter-frequency cell re-selection

Table H.2.2-1: SIB2: NR inter frequency cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionInfoCommon SEQUENCE { |  |  |  |
| rangeToBestCell | Not present |  |  |
| } |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearchP | 25 | 50 is actual value in dB (25 * 2 dB) |  |
| threshServingLowP | 22 | 44 is actual value in dB (22 * 2 dB) |  |
| cellReselectionPriority | 4 |  | NR Cell 1 |
|  | 5 |  | NR Cell 2 |
| } |  |  |  |
| } |  |  |  |

SIB4: for NR inter-frequency cell re-selection

Table H.2.2-2: SIB4: NR inter frequency cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB4 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 1 Entry |  |  |
| dl-CarrierFreq[1] | Downlink NR SSB ARFCN of NR Cell 2 |  | NR Cell 1 |
|  | Downlink NR SSB ARFCN of NR Cell 1 |  | NR Cell 2 |
| smtc | SSB-MTC specified in TS 38.508-1 [14] Table 7.3.1-3 with condition SMTC.n |  | SMTC.n |
| deriveSSB-IndexFromCell[1] | false |  | Asynchronous cells |
|  | true |  | Synchronous cells |
| q-RxLevMin[1] | -70 | -140 is actual value in dBm (-70 * 2 dBm) | dBm/15kHz |
|  | -69 | -137 is actual value in dBm (-69 * 2 +1 dBm) | dBm/30kHz |
| threshX-HighP[1] | 24 | 48 is actual value in dB (24 * 2 dB) |  |
| threshX-LowP[1] | 25 | 50 is actual value in dB (25 * 2 dB) |  |
| cellReselectionPriority[1] | 5 |  | NR Cell 1 |
|  | 4 |  | NR Cell 2 |
| q-OffsetFreq[1] | dB0 | 0 is actual value in dB (0 * 2 dB). |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB.n FRm | SSB pattern n in FRm according to TS 38.133 [6] A.3.10 |
| Synchronous cells | SSB indices of neighbour cells can be derived from timing of serving cell |
| Asynchronous cells | SSB indices of neighbour cells cannot be derived from timing of serving cell |

SIB1: for NR inter frequency cell re-selection

Table H.2.2-3: SIB1: NR inter frequency cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |  |
| q-RxLevMin |  | -70 | -140 is actual value in dBm (-70 * 2 dBm) | dBm/15kHz |
|  |  | -69 | -137 is actual value in dBm (-69 * 2 +1 dBm) | dBm/30kHz |
| } |  |  |  |  |
| } |  |  |  |  |

## H.2.3	System information blocks message contents exceptions for NR inter-RAT cell re-selection

SIB1: for inter-RAT NR – E-UTRA cell re-selection

Table H.2.3-1: SIB1: Inter-RAT NR – E-UTRA cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |  |
| q-RxLevMin |  | -70 | -140 is actual value in dBm (-70 * 2 dBm) | SCS15 |
|  |  | -69 | -137 is actual value in dBm (-69 * 2 +1 dBm) | SCS30 |
| } |  |  |  |  |
| } |  |  |  |  |

SIB2: for inter-RAT NR – E-UTRA cell re-selection

Table H.2.3-2: SIB2: Inter-RAT NR – E-UTRA cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearchP | 25 | 50 is actual value in dB (25 * 2 dB) |  |
| threshServingLowP | 22 | 44 is actual value in dB (22 * 2 dB) |  |
| cellReselectionPriority | 4 |  | higher priority |
|  | 5 |  | lower priority |
| } |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| smtc | SSB-MTC specified in TS 38.508-1 [14] Table 7.3.1-3 with condition SMTC.n |  | SMTC.n |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SMTC.n | SMTC pattern n according to TS 38.133 [6] A.3.11 |
| higher priority | NR cell re-selection to higher priority E-UTRA |
| lower priority | NR cell re-selection to lower priority E-UTRA |

SIB5: for inter-RAT NR – E-UTRA cell re-selection

Table H.2.3-3: SIB5: Inter-RAT NR – E-UTRA cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB5 ::= SEQUENCE { |  |  |  |
| carrierFreqListEUTRA SEQUENCE (SIZE (1..maxEUTRA-Carrier)) OF SEQUENCE { | 1 Entry |  |  |
| carrierFreq[1] | Downlink EUTRA ARFCN of E-UTRA Cell 1 |  |  |
| eutra-FreqNeighCellList[1] SEQUENCE (SIZE (1..maxCellEUTRA)) OF SEQUENCE { |  |  |  |
| physCellId | E-UTRA physical cell identity |  |  |
| dummy | dB0 |  |  |
| } |  |  |  |
| presenceAntennaPort1[1] | FALSE |  |  |
| cellReselectionPriority[1] | 5 |  | higher priority |
|  | 4 |  | lower priority |
| threshX-High | 24 | 48 is actual value in dB (24 * 2 dB) |  |
| threshX-Low | 25 | 50 is actual value in dB (25 * 2 dB) |  |
| q-RxLevMin | -70 | -140 is actual value in dBm (-70 * 2 dBm) |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| higher priority | NR cell re-selection to higher priority E-UTRA |
| lower priority | NR cell re-selection to lower priority E-UTRA |

# H.3	RRC message content exceptions

## H.3.1	RRC messages and information elements contents exceptions for NR measurement configuration

RRCReconfiguration

To setup NR Measurement Configuration.

Table H.3.1-1: RRCReconfiguration: NR measurement Configuration

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-13 with condition NR_MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| measConfig |  | MeasConfig-DEFAULT | Measurements configuration |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

MeasConfig-DEFAULT

Configuration for NR measurement.

Table H.3.1-2: MeasConfig-DEFAULT: Configuration of NR measurement

| Derivation path: 38.508-1 [14] table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF SEQUENCE { | n entries | n=1 without condition n=2 for INTER-FREQ OR INTER-RAT OR Deactivated SCell |  |
| measObjectId[1] | 1 |  |  |
| measObject[1] CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT with Condition INTRA-FREQ MO |  |  |
| } |  |  |  |
| measObjectId[2] | 2 |  | INTER-FREQ OR INTER-RAT OR Deactivated SCell |
| measObject[2] CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT with Condition INTER-FREQ MO |  | INTER-FREQ |
| measObjectNR | MeasObjectNR-DEFAULT with Condition Deactivated SCell |  | Deactivated SCell |
| measObjectEUTRA | MeasObjectEUTRA-DEFAULT |  | INTER-RAT |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | ReportConfigId |  |  |
| reportConfig[1] CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-DEFAULT |  |  |
| reportConfigInterRAT | ReportConfigInterRAT-DEFAULT |  | INTER-RAT |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | MeasId |  |  |
| measObjectId[1] | 1 |  |  |
|  | 2 |  | INTER-FREQ OR INTER-RAT OR Deactivated SCell |
| reportConfigId[1] | ReportConfigId |  |  |
| } |  |  |  |
| quantityConfig | QuantityConfig-DEFAULT |  |  |
|  | QuantityConfig | TS 38.508-1 Table 4.6.3-127 | L3 FILTERING NEEDED |
| measGapConfig | MeasGapConfig-DEFAULT |  | GAP NEEDED |
| } |  |  |  |

| Condition | Explanation |  |
| --- | --- | --- |
| GAP NEEDED | Measurement gap on the NR Cell is needed for measurement |  |
| INTER-FREQ | Configuration for inter-frequency NR measurement tests |  |
| INTER-RAT | Configuration for inter-RAT EUTRA measurement tests |  |
| Deactivated SCell |  | Configuration for measurement on deactivated SCell tests |
| L3 FILTERING NEEDED |  | L3 filtering is needed for measurement |

MeasObjectNR-DEFAULT

NR measurement object configuration.

Table H.3.1-3: MeasObjectNR-DEFAULT: NR intra-frequency measurement object configuration

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR of the SSB associated to serving cell | frequency of the serving cell | INTRA-FREQ MO |
|  | ARFCN-ValueNR of the SSB associated to SCell |  | Deactivated SCell |
|  | ARFCN-ValueNR of the SSB associated to inter-frequency neighbour cell |  | INTER-FREQ MO |
| referenceSignalConfig SEQUENCE { |  |  |  |
| ssb-ConfigMobility SEQUENCE { |  |  |  |
| ssb-ToMeasure | Not present |  |  |
| deriveSSB-IndexFromCell | false |  | Asynchronous cells |
|  | true |  | Synchronous cells OR RLM |
| } |  |  |  |
| } |  |  |  |
| absThreshSS-BlocksConsolidation SEQUENCE { |  |  |  |
| thresholdRSRP | 0 | SS-RSRP < -156dB | NOT SS-SINR |
| thresholdSINR | 0 | SS-SINR<-23 | SS-SINR |
| } |  |  |  |
| measCycleSCell-v1530 | sf640 |  | Deactivated SCell |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| INTRA-FREQ MO | Configuration for NR MO associated to intra-frequency carrier |
| INTER-FREQ MO | Configuration for NR MO associated to inter-frequency carrier |
| Synchronous cells | SSB indices of neighbourcells can be derived from timing of serving cell |
| Asynchronous cells | SSB indices of neighbourcells cannot be derived from timing of serving cell |
| RLM | Configuration for RLM tests |
| Deactivated SCell | Configuration for SCell activation and deactivation tests |
| SS-SINR | Configuration for SS-SINR tests |

MeasObjectEUTRA-DEFAULT

EUTRA measurement object configuration for NR FR1 to E-UTRAN handover.

Table H.3.1-3A: MeasObjectEUTRA-DEFAULT: InterRAT EUTRA measurement object configuration for FR1 to E-UTRAN handover

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-74 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectEUTRA ::= SEQUENCE { |  |  |  |
| cellsToAddModListEUTRAN SEQUENCE (SIZE (1..maxCellMeasEUTRA)) OF SEQUENCE{ |  |  |  |
| cellIndexEUTRA | 1 |  |  |
| physCellId | Physical Cell ID of the E-UTRA cell | Annex E |  |
| cellIndividualOffset | dB0 |  |  |
| } |  |  |  |
| eutra-PresenceAntennaPort1 | false |  |  |
| } |  |  |  |

ReportConfigNR-DEFAULT

NR Report Configuration

Table H.3.1-4: ReportConfigNR-DEFAULT(a3-offset): NR report configuration for event A3

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition EVENT_A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | A3-offset*2 | The actual value is field value * 0.5 dB |  |
| } |  |  |  |
| hysteresis | 0 | 0 dB |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| reportQuantity-RsIndexes SEQUENCE { |  |  | SSB Index |
| rsrp | true |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| maxNrofRS-IndexesToReport | 2 |  | SSB Index |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB Index | To include SSB Index |

Table H.3.1-4AA: ReportConfigNR-DEFAULT(a4-threshold): NR report configuration for event A4

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition EVENT_A4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA4 SEQUENCE { |  |  |  |
| a4-Threshold CHOICE { |  |  |  |
| rsrp | a4-threshold+156 | For thresholds, the actual value is (IE value – 156) dBm |  |
| } |  |  |  |
| hysteresis | 0 |  |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| reportQuantity-RsIndexes SEQUENCE { |  |  | SSB Index |
| rsrp | true |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| maxNrofRS-IndexesToReport | 2 |  | SSB Index |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB Index | To include SSB Index |

ReportConfigInterRAT-DEFAULT

InterRAT NR Report Configuration for NR FR1 to E-UTRAN handover.

Table H.3.1-4A: ReportConfigInterRAT- DEFAULT (b2-Thres1, b2-Thres2): InterRAT NR report configuration for FR1 to E-UTRAN handover with b2-Threshold1 = b2-Thres1 and b2-Threshold2EUTRA  = b2-Thres2 dBm

| Derivation Path: 38.508-1 [4] Table 4.6.3-141 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigInterRAT ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventB2 SEQUENCE { |  |  |  |
| b2-Threshold1 CHOICE { |  |  |  |
| rsrp | b2-Thres1 | b2-Thres1 – 156 dBm |  |
| } |  |  |  |
| b2-Threshold2EUTRA CHOICE { |  |  |  |
| rsrp | b2-Thres2 | b2-Thres2 – 140 dBm |  |
| } |  |  |  |
| hysteresis | 0 | 0 dB |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| reportAmount | infinity |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | true |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

QuantityConfig-DEFAULT

NR quantity configuration when L3 filtering is not used.

Table H.3.1-5: QuantityConfig-DEFAULT: NR quantity configuration when L3 filtering is not used

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-127 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QuantityConfig::= SEQUENCE { |  |  |  |
| quantityConfigNR-List SEQUENCE (SIZE (1..maxNrofQuantityConfig)) OF SEQUENCE { | 1 entry |  |  |
| quantityConfigCell[1] SEQUENCE { |  |  |  |
| ssb-FilterConfig SEQUENCE { |  |  |  |
| filterCoefficientRSRP | fc0 | No L3 filtering |  |
| filterCoefficientRSRQ | fc0 | No L3 filtering |  |
| filterCoefficientRS-SINR | fc0 | No L3 filtering |  |
| } |  |  |  |
| csi-RS-FilterConfig SEQUENCE { |  |  |  |
| filterCoefficientRSRP | fc0 | No L3 filtering |  |
| filterCoefficientRSRQ | fc0 | No L3 filtering |  |
| filterCoefficientRS-SINR | fc0 | No L3 filtering |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| quantityConfigEUTRA | Not present |  |  |
| quantityConfigEUTRA SEQUENCE { |  |  | INTER-RAT |
| filterCoefficientRSRP | fc0 |  |  |
| filterCoefficientRSRQ | fc0 |  |  |
| filterCoefficientRS-SINR | fc0 |  |  |

| Condition | Explanation |
| --- | --- |
| INTER-RAT | Configuration for inter-RAT EUTRA measurement tests |

MeasGapConfig-DEFAULT

Measurement gap configuration.

Table H.3.1-6: MeasGapConfig: measurement gap configuration

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-70 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapConfig ::= SEQUENCE { |  |  |  |
| gapFR2 CHOICE { |  |  | gapFR2 |
| setup SEQUENCE { |  |  |  |
| gapOffset | 39 |  | Pattern #13 |
| mgl | ms5dot5 |  | Pattern #13 |
| mgrp | ms40 |  | Pattern #13 |
| mgta | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| gapFR1 CHOICE { |  |  | gapFR1 |
| setup SEQUENCE { |  |  |  |
| gapOffset | 39 |  | Pattern #2 |
|  | 19 |  | Pattern #4 |
| mgl | ms3 |  | Pattern #2 |
|  | ms6 |  | Pattern #4 |
| mgrp | ms40 |  | Pattern #2 |
|  | ms20 |  | Pattern #4 |
| mgta | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| gapUE CHOICE { |  |  | gapUE |
| setup SEQUENCE { |  |  |  |
| gapOffset | 39 |  | Pattern #0 OR Pattern #2 OR Pattern #13 |
|  | 0 |  | RLM OR BFD |
| mgl | ms6 |  | Pattern #0 OR RLM OR BFD |
|  | ms3 |  | Pattern #2 |
|  | ms5dot5 |  | Pattern #13 |
| mgrp | ms40 |  | Pattern #0 OR Pattern #2 OR Pattern #13 OR RLM OR BFD |
| mgta | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Pattern #0 | Measurement gap pattern #0 defined in TS 38.133 [6] Table 9.1.2-1 is used for measurement tests |
| Pattern #2 | Measurement gap pattern #2 defined in TS 38.133 [6] Table 9.1.2-1 is used for measurement tests |
| Pattern #4 | Measurement gap pattern #4 defined in TS 38.133 [6] Table 9.1.2-1 is used for measurement tests |
| Pattern #13 | Measurement gap pattern #13 defined in TS 38.133 [6] Table 9.1.2-1 is used for measurement tests |
| RLM | Measurement gap pattern for RLM tests |
| BFD | Measurement gap pattern for Beam Failure Dectection tests |
| gapFR2 | Indicates measurement gap configuration that applies to FR2 only. gapFR2 cannot be configured together with gapUE. In (NG)EN-DC or NE-DC, gapFR2 can only be set up by NR RRC |
| gapFR1 | Indicates measurement gap configuration that applies to FR1 only. gapFR1 cannot be configured together with gapUE. In (NG)EN-DC, gapFR1 cannot be set up by NR RRC |
| gapUE | Indicates measurement gap configuration that applies to all frequencies (FR1 and FR2). If gapUE is configured, then neither gapFR1 nor gapFR2 can be configured. In (NG)EN-DC, gapUE cannot be set up by NR RRC. |

MeasResults-DEFAULT

measurement result for NR measurements.

Table H.3.1-7: MeasResults: measurement result for NR measurements

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-79 with condition A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| measResults SEQUENCE { |  |  |  |  |
| measId |  | MeasId |  |  |
| measResultServingMOList SEQUENCE (SIZE (1..maxNrofServingCells)) OF SEQUENCE { |  | 2 entries |  |  |
| servCellId[1] |  | ServCellIndex of NR SpCell |  |  |
| measResultServingCell[1] SEQUENCE { |  |  |  |  |
| physCellId |  | PhysCellId of NR SpCell |  |  |
| measResult SEQUENCE { |  |  |  |  |
| cellResults SEQUENCE { |  |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |  |
| rsrp |  | (0..127) |  |  |
| rsrq |  | (0..127) |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| servCellId[2] |  | ServCellIndex of NR SCell |  | Deactivated SCell |
| measResultServingCell[2] SEQUENCE { |  |  |  | Deactivated SCell |
| physCellId |  | PhysCellId of NR SCell |  |  |
| measResult SEQUENCE { |  |  |  |  |
| cellResults SEQUENCE { |  |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |  |
| rsrp |  | (0..127) |  |  |
| rsrq |  | (0..127) |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |  |
| measResultListNR SEQUENCE (SIZE(1..maxCellReport)) OF SEQUENCE { |  | 1 entry |  | INTRA-FREQ OR INTER-FREQ |
| physCellId[1] |  | PhysCellId of NR neighbour Cell |  |  |
| measResult[1] SEQUENCE { |  |  |  |  |
| cellResults SEQUENCE { |  |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |  |
| rsrp |  | (0..127) |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| rsIndexResults SEQUENCE { |  | n entires of ResultsPerSSB-Index | ResultsPerSSB-IndexList | SSB Index |
| resultsPerSSB-Index SEQUENCE { |  | entry [1] |  |  |
| ssb-Index |  | SSB-Index | an SS-Block within an SS-Burst |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { |  | 1 entry |  | INTER-RAT |
| eutra-PhysCellId [1] |  | PhysCellId of E-UTRA neighbour Cell |  |  |
| measResult[1] SEQUENCE { |  |  |  |  |
| rsrp |  | (0..97) |  |  |
| rsrq |  | (0..34) |  |  |
| } |  |  |  |  |
| cgi-Info |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB Index | To include SSB Index |
| INTRA-FREQ | Configuration for intra-frequency NR measurement tests |
| INTER-FREQ | Configuration for inter-frequency NR measurement tests |
| INTER-RAT | Configuration for inter-RAT EUTRA measurement tests |
| Deactivated SCell | Configuration for measurement on Deactivated SCell tests |

RadioLinkMonitoringConfig -DEFAULT

Default configuration for RLM resources.

Table H.3.1-8: RadioLinkMonitoringConfig-DEFAULT: Default configuration for RLM and BFD resources

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF { | n entries | n = 1 for FR1, n = 2 for FR2 | SSB RLM OR CSI-RS RLM |
| RadioLinkMonitoringRS[1] SEQUENCE { |  |  |  |
| radioLinkMonitoringRS-Id | 0 |  |  |
| purpose | rlf |  |  |
| detectionResource CHOICE { |  |  |  |
| ssb-Index | 0 | Index of SSB #0 | SSB RLM |
| csi-RS-Index | NZP-CSI-RS-ResourceId for TRS(4) | TS 38.508-1[14], table 7.3.1-7C | CSI-RS RLM |
| } |  |  |  |
| } |  |  |  |
| RadioLinkMonitoringRS[2] SEQUENCE { |  |  | FR2 |
| radioLinkMonitoringRS-Id | 1 |  |  |
| purpose | rlf |  |  |
| detectionResource CHOICE { |  |  |  |
| ssb-Index | 1 | Index of SSB #1 | SSB RLM |
| csi-RS-Index | NZP-CSI-RS-ResourceId for TRS(4) with condition SECOND_SET | TS 38.508-1[14], table 7.3.1-7C | CSI-RS RLM |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF { | 2 entries |  | SSB BFD OR CSI-RS BFD |
| RadioLinkMonitoringRS[1] SEQUENCE { |  |  |  |
| radioLinkMonitoringRS-Id | 0 |  |  |
| purpose | both |  |  |
| detectionResource CHOICE { |  |  |  |
| ssb-Index | 0 | Index of SSB #0 | SSB BFD |
| csi-RS-Index | NZP-CSI-RS-ResourceId for BM(0) | TS 38.508-1[14], Table 7.3.1-7D | CSI-RS BFD |
| } |  |  |  |
| } |  |  |  |
| RadioLinkMonitoringRS[2] SEQUENCE { |  |  |  |
| radioLinkMonitoringRS-Id | 1 |  |  |
| purpose | rlf |  |  |
| detectionResource CHOICE { |  |  |  |
| ssb-Index | 1 | Index of SSB #1 | SSB BFD |
| csi-RS-Index | NZP-CSI-RS-ResourceId for BM(1) | TS 38.508-1[14], Table 7.3.1-7D | CSI-RS BFD |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| beamFailureInstanceMaxCount | n1 |  | SSB BFD OR CSI-RS BFD |
| beamFailureDetectionTimer | pbfd4 |  | SSB BFD OR CSI-RS BFD |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB RLM | Used when SSB based RLM is configured in test case |
| CSI-RS RLM | Used when CSI-RS based RLM is configured in test case |
| SSB BFD | Used when SSB based BFD is configured in test case |
| CSI-RS BFD | Used when CSI-RS based BFD is configured in test case |

RLF-TimersAndConstants-DEFAULT

Default parameters for RLM related timers and counters.

Table H.3.1-9: RLF-TimersAndConstants-DEFAULT

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-150 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLF-TimersAndConstants ::= SEQUENCE { |  |  |  |
| t310 | ms0 |  |  |
| } |  |  |  |

BeamFailureRecoveryConfig-DEFAULT

Default configuration for CBD and contention-free RACH in link recovery.

Table H.3.1-10: BeamFailureRecoveryConfig-DEFAULT

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BeamFailureRecoveryConfig ::= SEQUENCE { |  |  |  |
| rootSequenceIndex-BFR | 0 |  |  |
| rach-ConfigBFR | RACH-ConfigGeneric-DEFAULT |  | SSB |
|  | RACH-ConfigGeneric-DEFAULT with condition CSI-RS |  | CSI-RS |
| rsrp-ThresholdSSB | 58 | -98dBm | SCS15 |
|  | 61 | -95dBm | SCS30 |
| candidateBeamRSList SEQUENCE (SIZE(1..maxNrofCandidateBeams)) OF CHOICE { | 1 entry |  |  |
| ssb[1] SEQUENCE { |  |  | SSB |
| ssb | 1 |  |  |
| ra-PreambleIndex | 50 |  |  |
| } |  |  |  |
| csi-RS[1] SEQUENCE { |  |  | CSI-RS |
| csi-RS | NZP-CSI-RS-ResourceId of the CSI-RS resource used for CBD |  |  |
| ra-OccasionList | 1 |  |  |
| ra-PreambleIndex | 50 |  |  |
| } |  |  |  |
| } |  |  |  |
| ssb-perRACH-Occasion | oneFourth |  |  |
| ra-ssb-OccasionMaskIndex | Not present |  |  |
| recoverySearchSpaceId | SearchSpaceId of the search space used for BFD RAR in DL active BWP |  |  |
| ra-Prioritization | Not present |  |  |
| beamFailureRecoveryTimer | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB | Configuration for SSB based CBD |
| CSI-RS | Configuration for CSI-RS based CBD |

BWP-UplinkDedicated

Default BWP configuration for Beam Failure Recovery

Table H.3.1-10A: BWP-UplinkDedicated

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicated ::= SEQUENCE { |  |  |  |
| beamFailureRecoveryConfig | BeamFailureRecoveryConfig-DEFAULT |  |  |
| } |  |  |  |

RACH-ConfigGeneric-DEFAULT

Default generic configuration for contention-free RACH in link recovery.

Table H.3.1-11: RACH-ConfigGeneric-DEFAULT

| Derivation Path: TS 38.508-1 [14], Table 7.3.1-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| preambleTransMax | n200 |  | CSI-RS |
| ra-ResponseWindow | sl40 |  | FR2 AND CSI-RS |
|  | sl1 |  | FR1 AND CSI-RS |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| CSI-RS | Configuration for CSI-RS based CBD |

## H.3.2	RRC messages and information elements contents exceptions for NR cell re-selection and handoverRACH-ConfigGeneric: for NR cell re-selection and handover

Table H.3.2-1: Void

Table H.3.2-2: RRCReconfiguration-HO

| Derivation Path: TS 38.508-1, table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig with conditions SRB1 and SRB2 and DRBn and Re-establish_PDCP |  | RBConfig_KeyChange |
|  | RadioBearerConfig with conditions DRBn and Recover_PDCP |  | RBConfig_NoKeyChange |
| secondaryCellGroup | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig with conditions PCell_change | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| RBConfig_KeyChange | RadioBearerConfig to perform Intra-NR handover with security key change |
| RBConfig _NoKeyChange | RadioBearerConfig to perform Intra-NR handover without security key change |

## H.3.3	RRC messages and information elements contents exceptions for NR inter-RAT handover

MobilityFromNRCommand

For Inter-RAT NR handover.

Table H.3.3-1: MobilityFromNRCommand: InterRAT NR handover

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MobilityFromNRCommand::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| mobilityFromNRCommand SEQUENCE { |  |  |  |  |
| targetRAT-Type |  | eutra |  |  |
| targetRAT-MessageContainer |  | OCTET STRING including the RRCConnectionReconfiguration message according TS 36.508 [2], table 4.6.1-8 with condition HO-TO-EUTRA |  |  |
| nas-SecurityParamFromNR |  | The 4 LSB of the downlink NAS COUNT |  |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

MobilityFromEUTRACommand

Table H.3.3-2: MobilityFromEUTRACommand: inter-RAT handover to NR Cell

| Derivation Path:TS 36.508 [25] Table 4.6.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MobilityFromEUTRACommand ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| mobilityFromEUTRACommand-r9 SEQUENCE { |  |  |  |  |
| purpose CHOICE { |  |  |  |  |
| handover SEQUENCE { |  |  |  |  |
| targetRAT-Type |  | nr |  |  |
| targetRAT-MessageContainer |  | OCTET STRING containing RRCReconfiguration according to TS 38.508-1 [14] with Condition NR |  |  |
| nas-SecurityParamFromEUTRA |  | Not present |  |  |
| systemInformation |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

## H.3.4	E-UTRA RRC messages and information elements contents exceptions for NR measurement configuration

RRCConnectionReconfiguration

Includes the nr-SecondaryCellGroupConfig-r15 to convey NR RRCReconfiguration message as specified in TS 38.331 [13].

Table H.3.4-1: RRCConnectionReconfiguration: NR RRC Reconfiguration in EN-DC

| Derivation Path: 36.508 [25], Table 4.6.1-8 with condition MCG_and_SCG |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |  |  |  |
| rrcConnectionReconfiguration-r8 ::= SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |  |  |  |
| nr-SecondaryCellGroupConfig-r15 |  | RRCReconfiguration |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table H.3.4-1a: RRCConnectionReconfiguration for measurement configuration

For measurement configuration provided by NR PSCell under EN-DC.

| Derivation Path: 36.508 [25], Table 4.6.1-8 with condition MEAS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |  |  |  |
| rrcConnectionReconfiguration-r8 ::= SEQUENCE { |  |  |  |  |  |  |  |
| measConfig |  | Not present |  |  |  |  |  |  |
|  |  | MeasConfig-DEFAULT |  | Table H.3.4-4 |  | GAP_NEEDED |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |  |  |  |
| nr-SecondaryCellGroupConfig-r15 |  | RRCReconfiguration |  | Table H.3.1-1 |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| GAP_NEEDED | Measurement gap is needed in Test |

ULInformationTransferMRDC

uplink transfer of MR DC information for transferring the NR RRC Measurement Report message in EN-DC.

Table H.3.4-2: ULInformationTransferMRDC: uplink transfer of MR DC information

| Derivation Path: 36.508 [25], Table 4.6.1-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ULInformationTransferMRDC ::= SEQUENCE { |  |  |  |
| ul-DCCH-MessageNR-r15 | OCTET STRING including the MeasurementReport |  |  |
| } |  |  |  |

RRCConnectionReconfigurationComplete

Includes the scg-ConfigResponseNR to convey NR RRCReconfigurationComplete message as specified in TS 38.331 [13].

Table H.3.4-3: RRCConnectionReconfigurationComplete: NR RRC Reconfiguration Complete in EN-DC

| Derivation Path: 36.508 [25], Table 4.6.1-9: with condition MCG_and_SCG |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| RRCConnectionReconfigurationComplete ::= SEQUENCE { |  |  |  |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-UL |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| rrcConnectionReconfigurationComplete-r8 SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| scg-ConfigResponseNR-r15 |  | OCTET STRING including the RRCReconfigurationComplete message according TS 38.508-1, table 4.6.1-14. |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

MeasConfig-DEFAULT

Configures measurement gap that applies to FR1 only in EN-DC as specified in TS 38.331 [13].

Table H.3.4-4: MeasConfig-DEFAULT

| Derivation Path: 36.508 [25],Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | 1 |  |  |
| measObject[1] CHOICE { |  |  |  |
| measObjectEUTRA | MeasObjectEUTRA-GENERIC(Freq) | Freq is the frequency of E-UTRA PCell |  |
| } |  |  |  |
| } |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  | INTER-RAT NR |
| measObjectId[1] | 1 |  |  |
| measObject[1] CHOICE { |  |  |  |
| measObjectEUTRA | MeasObjectEUTRA-GENERIC(Freq) | Freq is the frequency of E-UTRA PCell |  |
| } |  |  |  |
| measObjectId[2] | 2 |  |  |
| measObject[2] CHOICE { |  |  |  |
| measObjectNR-r15 | MeasObjectNR | Table H.3.4-6 |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  | INTER-RAT NR |
| reportConfigId[1] | 1 |  |  |
| reportConfig[1] CHOICE { |  |  |  |
| reportConfigEUTRA | ReportConfigEUTRA-A4(Thres) | Set Thres according to test parameters specified in test cases | EVENT A4 |
| reportConfigInterRAT | ReportConfigInterRAT-B2-NR(EUTRA-Thres, NR-Thres) | Set EUTRA-Thres and NR-Thres according to test parameters specified in test cases | EVENT B2 |
|  | ReportConfigInterRAT-B1-NR(EUTRA-Thres, NR-Thres) | Set EUTRA-Thres and NR-Thres according to test parameters specified in test cases | EVENT B1 |
|  | ReportConfigInterRAT-SFTD with Condition INTER-RAT | TS 36.508 [25] Table 4.6.6-8B | SFTD |
|  | ReportConfigInterRAT-PERIODICAL | Table 4.6.6-9 in TS 36.508 [25] | PERIODICAL |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  | INTER-RAT NR |
| measId[1] | 1 |  |  |
| measObjectId[1] | 2 |  |  |
| reportConfigId[1] | 1 |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig-FR1 | Table H.3.4-5 |  |
|  | Not present |  | GAPLESS |
| fr1-Gap-r15 | false |  | gapUE |
|  | true |  | gapFR1 |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| gapUE | fr1-Gap-r15 set to false indicates the gap is applicable for measurements on FR1 and FR2. E-UTRAN includes this field only when the UE is configured with (NG)EN-DC. |
| gapFR1 | fr1-Gap-r15 set to true indicates the gap is only applicable for measurements on FR1. E-UTRAN includes this field only when the UE is configured with (NG)EN-DC. |
| INTER-RAT NR | Measurement configuration for inter-RAT NR measurements |
| EVENT B2 | For event B2 triggered measurement reporting test cases |
| EVENT B1 | For event B1 triggered measurement reporting test cases |
| EVENT A4 | For event A4 triggered measurement reporting |
| SFTD | For inter-RAT SFTD measurement test cases |
| GAPLESS | MG is not needed in test |
| PERIODICAL | For periodic measurement reporting test cases |

MeasGapConfig-FR1

Configures FR1 measurement gap by LTE RRC in EN-DC as specified in TS 38.331 [13].

Table H.3.4-5: MeasGapConfig-FR1

| Derivation Path: 36.508 [25],Table 4.6.6-1A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapConfig ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| gapOffset CHOICE { |  |  |  |
| gp0 | 39 | TGRP = 40 ms | Pattern #0 |
|  | 0 |  | BFD |
| gp4-r15 | 19 | TGRP = 20 ms | Pattern #4 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Pattern #0 | gp0 corresponds to gap offset of Gap Pattern Id "0" with MGRP = 40ms |
| Pattern #4 | gp4-r15 corresponds to gap offset of Gap Pattern Id "4" with MGRP = 40ms (see TS 38.133, Table 9.1.2-1). It can be applied for (NG)EN-DC, see TS 38.133, Table 9.1.2-2. |
| BFD | Measurement gap pattern for Beam Failure Dectection tests |

Table H.3.4-6: MeasObjectNR

| Derivation Path: TS 36.508 [25] Table 4.6.6-2B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR-GENERIC(Freq) ::= SEQUENCE { |  |  |  |
| rs-ConfigSSB-r15 ::= SEQUENCE { |  |  |  |
| measTimingConfig-r15 ::= SEQUENCE { |  |  |  |
| periodicityAndOffset-r15 CHOICE { |  |  |  |
| sf20-r15 | 0 |  | SMTC.1 or SMTC.2 |
|  | 10 |  | SMTC.4 or SMTC.5 |
|  | 17 |  | SMTC.6 |
| sf160-r15 | 0 |  | SMTC.3 |
| } |  |  |  |
| ssb-Duration-r15 | sf1 |  | SMTC.1, SMTC.3 or SMTC.4 |
|  | sf5 |  | SMTC.2, SMTC.5 or SMTC.6 |
| } |  |  |  |
| subcarrierSpacingSSB-r15 | kHz15 |  | SSB.1 FR1, SSB.3 FR1 or SSB.5 FR1 |
|  | kHz30 |  | SSB.2 FR1, SSB.4 FR1 or SSB.6 FR1 |
|  | kHz120 |  | SSB.1 FR2, SSB.3 FR2, SSB.5 FR2 or SSB.7 FR2 |
|  | KHz240 |  | SSB.2 FR2, SSB.4 FR2, SSB.6 FR2 or SSB.8 FR2 |
| } |  |  |  |
| deriveSSB-IndexFromCell-r15 | true |  | Synchronous cells |
|  | false |  | Asynchronous cells |
| } |  |  |  |

Table H.3.4-7: RRCConnectionReconfiguration for iRAT measurement configuration

| Derivation Path: 36.508 [25], Table 4.6.1-8 with condition MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 ::= SEQUENCE { |  |  |  |
| measConfig | MeasConfig-DEFAULT with Conditions INTER RAT NR and PERIODICAL | Table H.3.4-4 |  |
|  | MeasConfig-DEFAULT with Conditions INTER RAT NR and SFTD | Table H.3.4-4 | INTER-RAT and SFTD |
|  | MeasConfig-DEFAULT with Conditions INTER RAT NR and EVENT B2 | Table H.3.4-4 | INTER-RAT and EVENT B2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

## H.3.5	RRC messages and information elements contents exceptions for NR radio link monitoring (RLM)

CSI-RS information elements contents exception for NR RLM SSB-Based test cases

Table H.3.5-1 to H.3.5-3: Void

Table H.3.5-4: CSI-ReportConfig

| Derivation Path: TS 38.508[14] Table 7.3.1-12F |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| reportConfigType CHOICE { |  |  |  |
| periodic SEQUENCE { |  |  |  |
| reportSlotConfig ::= CHOICE { |  |  |  |
| slots5 | 0 |  | SCS15kHz_FDD |
| slots5 | 2 |  | SCS15kHz_TDD |
| slots10 | 4 |  | SCS30kHz |
| } |  |  |  |
| pucch-CSI-ResourceList SEQUENCE (SIZE (1..maxNrofBWPs)) OF{ |  |  |  |
| PUCCH_CSI_Resource[0] SEQUENCE { |  |  |  |
| uplinkBandwidthPartId | BWP-Id |  |  |
| pucch_Resource | 9 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table H.3.5-5 to H.3.5-7: Void

BWP-DownlinkDedicated: to setup radioLinkMonitoringConfig Configuration

Table H.3.5-8: Void

RadioLinkMonitoringConfig: Configuration for RLM RS

Table H.3.5-9: RadioLinkMonitoringConfig

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF SEQUENCE { | 1 entry |  |  |
| radioLinkMonitoringRS-Id[1] | 0 |  |  |
| purpose[1] | rlf |  |  |
| detectionResource[1] CHOICE { |  |  |  |
| ssb-Index | 0 |  | SSB RLM |
| csi-RS-Index | NZP-CSI-RS-ResourceId for TRS(4) |  | CSI-RS RLM |
| } |  |  |  |
| } |  |  |  |
| failureDetectionResourcesToReleaseList | Not present |  |  |
| beamFailureInstanceMaxCount | Not present |  |  |
| beamFailureDetectionTimer | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB RLM | Configuration for SSB based RLM test cases |
| CSI-RS RLM | Configuration for CSI-RS based RLM test cases |

Table H.3.5-10: Void

## H.3.6	RRC messages and IE content exceptions for L1-RSRP measurement for beam reporting

ServingCellConfig: Default generic configuration for enabling CSI measurements and reporting

Table H.3.6-1: Void

CSI-ReportConfig: Default generic configuration for L1-RSRP measurements

Table H.3.6-2: CSI-ReportConfig

| Derivation Path: TS 38.508-1 [14], Table 7.3.1-12G |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| csi-IM-ResourcesForInterference | Not present |  |  |
| nzp-CSI-RS-ResourcesForInterference | Not present |  |  |
| reportConfigType CHOICE { |  |  |  |
| periodic SEQUENCE { |  |  | PERIODIC |
| reportSlotConfig CHOICE { |  |  |  |
| slot80 | 7 | Periodicity 80 slots and offset 7 | SCS30 |
|  | 2 | Periodicity 80 slots and offset 2 | SCS15 |
| slot320 | 4 | Periodicity 320 slots and offset 4 | FR2 |
| pucch-CSI-ResourceList SEQUENCE { | 1 entry |  |  |
| (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| uplinkBandwidthPartId | BWP-Id of active BWP |  |  |
| pucch-Resource | 8 | PUCCH-format2 as configured in TS 38.508-1 [14], Table 4.6.3-112 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportQuantity CHOICE { |  |  |  |
| ssb-Index-RSRP | NULL |  | SS-RSRP |
| cri-RSRP | NULL |  | CSI-RSRP |
| } |  |  |  |
| timeRestrictionForChannelMeasurements | configured |  |  |
| groupBasedBeamReporting CHOICE { |  |  |  |
| disabled SEQUENCE { |  |  |  |
| nrofReportedRS | n2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| PERIODIC | Configuration for periodic reporting |
| SS-RSRP | L1-RSRP measurement based on SSB |
| CSI-RSRP | L1-RSRP measurement based on CSI-RS |

CSI-ResourceConfig: Default generic resource configuration for L1-RSRP measurements

Table H.3.6-3: CSI-ResourceConfig

| Derivation Path: TS 38.508-1 [14], Table 7.3.1-12B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| nzp-CSI-RS-SSB SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF { | 1 entry |  | CSI-RS |
| NZP-CSI-RS-ResourceSetId[0] | 0 |  |  |
| } |  |  |  |
| csi-SSB-ResourceSetList SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSetsPerConfig)) OF { | 1 entry |  | SSB |
| CSI-SSB-ResourceSetId[0] | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB | Configuration for SSB based CSI |
| CSI-RS | Configuration for CSI-RS based CSI |

Table H.3.6-4 to H.3.6-9: Void

## H.3.6A	RRC messages and IE content exceptions for L1-SINR measurement for beam reporting

ServingCellConfig: Default generic configuration for enabling CSI measurements and reporting.

CSI-ReportConfig: Default generic configuration for L1-SINR measurements.

Table H.3.6A-1: CSI-ReportConfig

| Derivation Path: TS 38.508-1 [14], Table 7.3.1-12G |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| csi-IM-ResourcesForInterference | CSI-IM-ResourceConfigId-IMR |  | CSI-IM_IMR |
|  | CSI-IM-ResourceConfigId-IMR with condition APERIODIC |  | CSI-IM_IMR AND APERIODIC |
| nzp-CSI-RS-ResourcesForInterference | CSI-ResourceConfigId-IMR |  | CSI-RS_IMR |
|  | CSI-ResourceConfigId-IMR with condition APERIODIC |  | CSI-RS_IMR AND APERIODIC |
| reportConfigType CHOICE { |  |  |  |
| periodic SEQUENCE { |  |  | PERIODIC |
| reportSlotConfig CHOICE { |  |  |  |
| slot80 | 7 | Periodicity 80 slots and offset 7 | SCS30 |
|  | 2 | Periodicity 80 slots and offset 2 | SCS15 |
|  | 4 | Periodicity 80 slots and offset 4 | FR2 |
| pucch-CSI-ResourceList SEQUENCE { | 1 entry |  |  |
| (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| uplinkBandwidthPartId | BWP-Id of active BWP |  |  |
| pucch-Resource | 8 | PUCCH-format2 as configured in TS 38.508-1 [14], Table 4.6.3-112 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| aperiodic SEQUENCE { |  |  | APERIODIC |
| reportSlotOffsetList SEQUENCE (SIZE (1..maxNrofUL-Allocations)) OF { | 1 entry |  |  |
| INTEGER[1] | 26 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportQuantity CHOICE { |  |  |  |
| cri-SINR-r16 | NULL |  | CSI-SINR |
| ssb-Index- SINR-r16 | NULL |  | SS-SINR |
| } |  |  |  |
| timeRestrictionForChannelMeasurements | configured |  |  |
| groupBasedBeamReporting CHOICE { |  |  |  |
| disabled SEQUENCE { |  |  |  |
| nrofReportedRS | n2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| PERIODIC | Configuration for periodic reporting |
| APERIODIC | Configuration for aperiodic reporting |
| SS-SINR | L1-SINR measurement based on SSB |
| CSI- SINR | L1-SINR measurement based on CSI-RS |
| CSI-RS_IMR | Configuration for NZP-CSI-RS based IMR |
| CSI-IM_IMR | Configuration for CSI-IM based IMR |

CSI-ResourceConfig for CMR: Default generic resource configuration for L1-SINR measurements.

Table H.3A.6-2: CSI-ResourceConfig for CMR

| Derivation Path: TS 38.508-1 [14], Table 7.3.1-12B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| nzp-CSI-RS-SSB SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF { | 1 entry |  | CSI-RS |
| NZP-CSI-RS-ResourceSetId[1] | 0 |  |  |
| } |  |  |  |
| csi-SSB-ResourceSetList SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSetsPerConfig)) OF { | 1 entry |  | SSB |
| CSI-SSB-ResourceSetId[1] | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | BWP-Id of active BWP |  |  |
| resourceType | aperiodic |  | APERIODIC |
|  | periodic |  | PERIODIC |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB | Configuration for SSB based CMR |
| CSI-RS | Configuration for CSI-RS based CMR |

CSI-ResourceConfig for IMR: For NZP-CSI-RS resource configured as dedicated IMR, refer to Table H.3A.6-3; For CSI-IM resource configured as dedicated IMR, refer to Table H.3A.6-4.

Table H.3A.6-3: CSI-ResourceConfig for NZP-CSI-RS as IMR

| Derivation Path: TS 38.508-1 [14], Table 7.3.1-12B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-ResourceConfigId | CSI-ResourceConfigId-IMR |  |  |
|  | CSI-ResourceConfigId-IMR with condition APERIODIC |  | APERIODIC |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| nzp-CSI-RS-SSB SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSetId[1] | NZP-CSI-RS-ResourceSetId-IMR |  |  |
|  | NZP-CSI-RS-ResourceSetId-IMR with condition APERIODIC |  | APERIODIC |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | BWP-Id of active BWP |  |  |
| resourceType | aperiodic |  | APERIODIC |
|  | periodic |  | PERIODIC |
| } |  |  |  |

Table H.3A.6-3a: NZP-CSI-RS-ResourceSet for IMR

| Derivation Path: TS 38.508-1 [14], Table 7.3.1-11B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSet ::= SEQUENCE { |  |  |  |
| nzp-CSI-ResourceSetId | NZP-CSI-RS-ResourceSetId-IMR |  |  |
|  | NZP-CSI-RS-ResourceSetId-IMR with condition APERIODIC |  | APERIODIC |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 2 entries |  |  |
| NZP-CSI-RS-ResourceId[1] | NZP-CSI-RS-ResourceId for IMR(0) | INTEGER 12 |  |
|  | NZP-CSI-RS-ResourceId for IMR(0) with condition APERIODIC | INTEGER 22 | APERIODIC |
| NZP-CSI-RS-ResourceId[2] | NZP-CSI-RS-ResourceId for IMR(1) | INTEGER 13 |  |
|  | NZP-CSI-RS-ResourceId for IMR(1) with condition APERIODIC | INTEGER 23 | APERIODIC |
| } |  |  |  |
| aperiodicTriggeringOffset | 6 |  | APERIODIC |
| } |  |  |  |

Table H.3A.6-3b: NZP-CSI-RS-Resource for IMR(Id)

| Derivation Path: TS 38.508-1 [14], Table 7.3.1-7B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceId | NZP-CSI-RS-ResourceId for IMR(Id) |  |  |
|  | NZP-CSI-RS-ResourceId for IMR(Id) with condition APERIODIC |  | APERIODIC |
| resourceMapping SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| row1 | 0001 |  |  |
| } |  |  |  |
| nrofPorts | p1 |  |  |
| firstOFDMSymbolInTimeDomain | 6 |  | Id = 0 |
|  | 10 |  | Id = 1 |
|  | 7 |  | Id = 0 AND APERIODIC |
|  | 11 |  | Id = 1 AND APERIODIC |
| cdm-Type | noCDM |  |  |
| density CHOICE { |  |  |  |
| three | NULL |  |  |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation-RRM |  | See TS 38.508-1 [14], Table 7.3.1-10 |
| } |  |  |  |
| powerControlOffset | 0 |  |  |
| powerControlOffsetSS | db0 |  |  |
| scramblingID | 0 |  |  |
| periodicityAndOffset { |  |  |  |
| slots20 | 1 |  | SCS15 |
| slots40 | 2 |  | SCS30 |
| } |  |  |  |
| periodicityAndOffset | Not present |  | APERIODIC |
| qcl-InfoPeriodicCSI-RS | Not present |  |  |
| } |  |  |  |

Table H.3A.6-3c: NZP-CSI-RS-ResourceSetId-IMR

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSetId | 1 |  |  |

Table H.3A.6-4: CSI-ResourceConfig for CSI-IM as IMR

| Derivation Path: TS 38.508-1 [14], Table 7.3.1-12B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-ResourceConfigId | CSI-IM-ResourceConfigId-IMR |  |  |
|  | CSI-IM-ResourceConfigId-IMR with condition APERIODIC |  | APERIODIC |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| csi-IM-ResourceSetList SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSetsPerConfig) OF { | 1 entry |  |  |
| CSI-IM-ResourceSetId[1] | CSI-IM-ResourceSetId-IMR |  |  |
|  | CSI-IM-ResourceSetId-IMR with condition APERIODIC |  | APERIODIC |
| } |  |  |  |
| } |  |  |  |
| bwp-Id | BWP-Id of active BWP |  |  |
| resourceType | aperiodic |  | APERIODIC |
|  | periodic |  | PERIODIC |
| } |  |  |  |

Table H.3A.6-4a: CSI-IM-ResourceSet for IMR

| Derivation Path: TS 38.508-1 [14], Table 7.3.1-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-ResourceSet ::= SEQUENCE { |  |  |  |
| csi-IM-ResourceSetId | CSI-IM-ResourceSetId-IMR |  |  |
|  | CSI-IM-ResourceSetId-IMR with condition APERIODIC |  | APERIODIC |
| csi-IM-Resources SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourcesPerSet)) OF CSI-IM-ResourceId { | 2 entries |  |  |
| CSI-IM-ResourceId[1] | CSI-IM-ResourceId for IMR(0) | INTEGER 0 |  |
|  | CSI-IM-ResourceId for IMR(0) with condition APERIODIC | INTEGER 10 | APERIODIC |
| CSI-IM-ResourceId[2] | CSI-IM-ResourceId for IMR(1) | INTEGER 1 |  |
|  | CSI-IM-ResourceId for IMR(1) with condition APERIODIC | INTEGER 11 | APERIODIC |
| } |  |  |  |
| } |  |  |  |

Table H.3A.6-4b: CSI-IM-Resource for IMR(Id)

| Derivation Path: TS 38.508-1 [14], Table 7.3.1-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-Resource ::= SEQUENCE { |  |  |  |
| csi-IM-ResourceId | CSI-IM-ResourceId for IMR(Id) |  |  |
|  | CSI-IM-ResourceId for IMR(Id) with condition APERIODIC |  | APERIODIC |
| csi-IM-ResourceElementPattern CHOICE { |  |  |  |
| pattern1 SEQUENCE { |  |  |  |
| subcarrierLocation-p1 | s0 |  |  |
| symbolLocation-p1 | 6 |  | Id = 0 |
|  | 10 |  | Id = 1 |
|  | 7 |  | Id = 0 AND APERIODIC |
|  | 11 |  | Id = 1 AND APERIODIC |
| } |  |  |  |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation-RRM |  | See TS 38.508-1 [14], Table 7.3.1-10 |
| periodicityAndOffset | CSI-ResourcePeriodicityAndOffset for CSI |  |  |
| slots20 | 1 |  | SCS15 |
| slots40 | 2 |  | SCS30 |
| } |  |  |  |
| periodicityAndOffset | Not present |  | APERIODIC |
| } |  |  |  |

Table H.3A.6-4c: CSI-IM-ResourceSetId-IMR

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-ResourceSetId | 0 |  |  |

## H.3.7	RRC messages and information elements contents exceptions for NR cell search when DRX is used

MAC-CellGroupConfig: DRX configuration for NR serving cell

Table H.3.7-1: MAC-CellGroupConfig: NR intra-frequency cell search when DRX is used

| Derivation Path: TS 38.508-1, Table 4.6.3-68 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MAC-CellGroupConfig ::= SEQUENCE { |  |  |  |
| drx-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| drx-onDurationTimer CHOICE { |  |  |  |
| milliSeconds | ms1 |  | DRX.1 OR DRX.2 OR DRX.6 |
|  | ms6 |  | DRX.3 OR DRX.7 OR DRX.8 OR DRX.11 |
| } |  |  |  |
| drx-InactivityTimer | ms1 |  |  |
| drx-RetransmissionTimerDL | sl1 |  |  |
| drx-RetransmissionTimerUL | sl1 |  |  |
| drx-LongCycleStartOffset CHOICE { |  |  |  |
| ms20 | 0 |  | DRX.11 |
| ms40 | 0 |  | DRX.1 or DRX.3 |
|  | 10 |  | (DRX.1 and Gap) OR (DRX.1 and Offset) |
|  | 15 |  | DRX.1 and Gap and INTER-FREQ |
|  | 20 |  | DRX.3 and Gap |
| ms320 | 0 |  | DRX.6 OR DRX.8 |
| ms640 | 0 |  | DRX.2 OR DRX.7 |
|  | 10 |  | (DRX.2 and Gap) OR (DRX.7 and Offset) |
|  | 15 |  | DRX.2 and Gap and INTER-FREQ |
| } |  |  |  |
| shortDRX | not present |  |  |
| drx-SlotOffset | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| tag-Config SEQUENCE { |  |  |  |
| tag-ToReleaseList | Not present |  |  |
| tag-ToAddModList SEQUENCE (SIZE (1..maxNrofTAGs)) OF SEQUENCE { | 1 entry |  |  |
| tag-Id[1] | 0 |  |  |
| timeAlignmentTimer[1] | ms500 |  | DRX.1 OR DRX.2 OR DRX.6 |
|  | infinity |  | DRX.3 OR DRX.7 OR DRX.8 OR DRX.11 |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| DRX.1 | DRX Configuration 1 according to TS 38.133 [6] A.3.3.1 |
| DRX.2 | DRX Configuration 2 according to TS 38.133 [6] A.3.3.2 |
| DRX.3 | DRX Configuration 3 according to TS 38.133 [6] A.3.3.3 |
| DRX.6 | DRX Configuration 6 according to TS 38.133 [6] A.3.3.6 |
| DRX.7 | DRX Configuration 7 according to TS 38.133 [6] A.3.3.7 |
| DRX.8 | DRX Configuration 8 according to TS 38.133 [6] A.3.3.8 |
| INTER-FREQ | Configuration for inter-frequency NR measurement tests |
| Gap | Configuration for tests with measurement gap |
| Offset | Tests that requires offset for DRX to eliminate overlap of DRX and SMTC |

MAC-MainConfig: DRX configuration for E-UTRAN serving cell

Table H.3.7-2: MAC-MainConfig

| Derivation Path: 36.508, Table 4.8.2.1.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| MAC-MainConfig ::= SEQUENCE { |  |  |  |  |
| drx-Config CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| onDurationTimer | psf2 |  | DRX.4 OR DRX.9 |  |
|  | psf6 |  | DRX.5 OR DRX.10 OR DRX.12 |  |
| drx-InactivityTimer | psf2 |  | DRX.4 OR DRX.12 |  |
|  | psf1920 |  | DRX.5 OR DRX.10 |  |
|  | psf100 |  | DRX.9 |  |
| longDRX-CycleStartOffset CHOICE { |  |  |  |  |
| sf160 | 0 |  | DRX.4 |  |
| sf320 | 0 |  | DRX.5 |  |
| sf40 | 0 |  | DRX.9 |  |
| sf640 | 0 |  | DRX.10 OR DRX.12 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| timeAlignmentTimerDedicated | infinity |  | DRX.4 OR DRX.5 OR DRX.12 |  |
|  | sf500 |  | DRX.9 OR DRX.10 |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| DRX.4 | DRX Configuration 4 according to TS 38.133 [6] A.3.3.4 |
| DRX.5 | DRX Configuration 5 according to TS 38.133 [6] A.3.3.5 |
| DRX.9 | DRX Configuration 9 according to TS 38.133 [6] A.3.3.9 |
| DRX.10 | DRX Configuration 10 according to TS 38.133 [6] A.3.3.10 |
| DRX.12 | DRX Configuration 12 according to TS 38.133 [6] A.3.3.12 |

## H.3.8	RRC messages and information elements contents exceptions for NR RRC reconfiguration delay

ServingCellConfigCommonSIB: information elements content exception for RRC reconfiguration delay test cases to specific both NR uplink and supplementary uplink for SCell.

Table H.3.8-1: ServingCellConfigCommonSIB-Procedure Delay

| Derivation Path: TS 38.508-1, table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| downlinkConfigCommon | DownlinkConfigCommonSIB |  |  |
| uplinkConfigCommon | UplinkConfigCommonSIB | NR uplink |  |
|  | UplinkConfigCommonSIB with condition SUL_NUL |  | SUL |
| supplementaryUplink | UplinkConfigCommonSIB with condition SUL_SUL |  | SUL |
| n-TimingAdvanceOffset | Not present |  |  |
| ssb-PositionsInBurst SEQUENCE { |  |  |  |
| inOneGroup | ’0100 0000’B | When carrier frequency is smaller than or equal to 3 GHz, only the 4 leftmost bits are valid; |  |
| groupPresence | Not present |  |  |
| } |  |  |  |
| ssb-PeriodicityServingCell | ms20 |  |  |
| tdd-UL-DL-ConfigurationCommon | TDD-UL-DL-ConfigCommon |  | FR1_TDD |
| ss-PBCH-BlockPower | 0 |  |  |
| } |  |  |  |

RRCReconfiguration: information elements contents exception for RRC reconfiguration delay test cases to specific radio bearers and cell groups.

Table H.3.8-2: RRCReconfiguration-Procedure Delay

| Derivation Path: TS 38.508-1, table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig with conditions SRB2 and DRB1 |  |  |
| } |  |  |  |
| secondaryCellGroup | CellGroupConfig with condition EN-DC_SCell_add | OCTET STRING (CONTAINING CellGroupConfig) |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig with conditions EN-DC | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

## H.3.9	RRC messages and information elements contents exceptions for UL timing

Table H.3.9-1: PUSCH-TimeDomainResourceAllocationList

| Derivation Path: TS 38.508-1, Table 4.6.3-122 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-TimeDomainResourceAllocationList ::= SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF { | 2 entries |  |  |
| PUSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  |  |  |
| startSymbolAndLength | 41 | Start symbol(S)=0, Length(L)=13 |  |
| } |  |  |  |
| PUSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  | addressed by Msg3 PUSCH time resource allocation field of the Random Access Response acc. to TS 38.213 [22] Table 8.2-1. |  |
| startSymbolAndLength | 41 | Start symbol(S)=0, Length(L)=13 |  |
| } |  |  |  |
| } |  |  |  |

Annex I (normative):
RRM OTA procedures

Annex I defines the RRM OTA procedures for the permitted testing methodologies defined in [32].

# I.0	Test applicability per permitted test method

The applicability of each permitted test method for the different RRM Angle of Arrival (AoA) setups as defined in Clause A.9 is defined in Table I.0-1.

Table I.0-1 AoA Test Setup applicability per permitted test method

| AoA Test Setup | D > 5cm or No declaration | D ≤ 5cm |
| --- | --- | --- |
| Setup 1 | IFF, Enhanced IFF | DFF, IFF, Enhanced IFF, IFF+DFF |
| Setup 2a | IFF, Enhanced IFF | DFF, IFF, Enhanced IFF, IFF+DFF |
| Setup 2b | IFF, Enhanced IFF | DFF, IFF, Enhanced IFF, IFF+DFF |
| Setup 3 | Enhanced IFF | DFF, Enhanced IFF, IFF+DFF |
| Setup 4a | Enhanced IFF | DFF, Enhanced IFF, IFF+DFF |
| Setup 4b | Enhanced IFF | DFF, Enhanced IFF, IFF+DFF |
| NOTE1: D =The diameter of the smallest sphere that encloses the radiating parts of the phase coherent array antenna(s) active at any one time during the test., declared by UE vendor as per Table 4.3.9-9 in TS 38.508-2 NOTE2: DFF indicates both DFF and DFF simplification in TR 38.810. NOTE3: For DFF and DFF part of IFF+DFF, minimum range length needs to meet the requirement as specified in 38.508-1 Annex B.2.2-4 with the declared D |  |  |

# I.1	Direct far field (DFF)

## I.1.1	RX beam peak direction search

The Rx beam peak direction search can be found using any of the following options:

-	Same measurement procedure as in clause Annex K.1.2 of TS 38.521-2 [18].

-	Any of the procedures described in Annex I.4.

## I.1.2	Search for directions corresponding to the EIS spherical coverage percentile

The test directions corresponding to the EIS spherical coverage percentile can be found using any of the following options:

-	EIS spherical coverage scan procedure as described in Annex K.1.6 of TS 38.521-2 [18].

-	Any of the procedures described in Annex I.4.

# I.2	Direct far field (DFF) simplification

## I.2.1	RX beam peak direction search

The Rx beam peak direction search can be found using any of the following options:

-	Same measurement procedure as in clause Annex K.2.2 of TS 38.521-2 [18].

-	Any of the procedures described in Annex I.4.

## I.2.2	Search for directions corresponding to the EIS spherical coverage percentile

The test directions corresponding to the EIS spherical coverage percentile can be found using any of the following options:

-	EIS spherical coverage scan procedure as described in Annex K.1.6 of TS 38.521-2 [18].

-	Any of the procedures described in Annex I.4.

# I.3	Indirect far field (IFF)

## I.3.1	RX beam peak direction search

The Rx beam peak direction search can be found using any of the following options:

-	Same measurement procedure as in clause Annex K.3.2 of TS 38.521-2 [18].

-	Any of the procedures described in Annex I.4.

## I.3.2	Search for directions corresponding to the EIS spherical coverage percentile

The test directions corresponding to the EIS spherical coverage percentile can be found using any of the following options:

-	EIS spherical coverage scan procedure as described in Annex K.1.6 of TS 38.521-2 [18].

-	Any of the procedures described in Annex I.4.

# I.3A	Enhanced indirect far field (Enhanced IFF)

## I.3A.1	RX beam peak direction search

Same as in clause I.3.1 for IFF.

## I.3A.2	Search for directions corresponding to the EIS spherical coverage percentile

Same as in clause I.3.2 for IFF.

# I.4	Procedures to search test directions for RRM FR2

## I.4.1	RSRPB-based scan with fallback option to Rx beam peak direction search

This clause provides RSRPB-based scan to find the Rx beam peak of the UE for Setup 1, single AoA cases and one of the test directions for Setup 4. It also provides a method to find test directions fulfilling the EIS Spherical Coverage (TS 38.521-2 [18] clause 7.3.4.3) for Setup 2, 3 and 4. Other approaches are not precluded. RSRPB-based peak beam scan greatly reduces the time to find the peak beam position compared to traditional EIS scan using either constant step size or constant density. The procedure can be used with DFF as well as IFF methodology. The procedure is achieved implementing the following steps:

1.	Enable periodic RSRPB reporting from the UE

2.	Set of grid points for the UE scan can be user defined set or entire sphere.

3.	Set the DL SSB_RP (at the centre of the QZ) to -95 dBm / SCS. For each grid point, record RSRPB first by connecting SS to the DUT through the measurement antenna with PolLink=  polarization to form the Rx beam towards the measurement antenna.

4.	Wait for BEAM_SELECT_WAIT_TIME before recording the RSRPB reports.

4a	For each of the grid points, calculate the RSRPB result for PolLink= as the linear sum of the 2 RSRPB reports (one per receiver branch).

4b	Repeat Steps 3, 4 and 4a with PolLink= .

5.	Once the grid points scan is completed, sort the grid points based on the linear sum of the RSRPB results for PolLink= and PolLink= .

6.	For the top 10 grid points, run the REFSENS throughput test as per the test condition defined in TS 38.521-2 [18] clause 7.3.2

7.	Select the grid point with the best REFSENS result among the 10 points from Step 6 as the Rx beam peak for RRM test cases using AoA Setup 1 or 4.

.

8.	Use the EIS result for the RRM beam peak to offset the RSRPB results to obtain an estimate of the EIS Spherical coverage (SC) map.

9.	From the EIS map in Step 8, select all the points meeting the EIS SC criteria (as defined in TS 38.521-2 [18] Clause 7.3.4.3) as potential test directions for AoA Setup 2a, 2b, 3 and 4. In order to confirm that the grid point is suitable, run the REFSENS throughput test as per the test condition defined in TS 38.521-2 [18] clause 7.3.2 and verify that the EIS SC criteria is met. .

If not enough grid points are found using this method, fallback to using the method in Annex I.1, I.2, I.3 or I.3A as applicable.

Annex J (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-08 | RAN5#76 | R5-175205 | - | - | - | Introduction of TS 38.533 | 0.0.1 |
| 2018-08 | RAN5#80 | R5-184115 | - | - | - | Added specification structure including headers up to third level | 0.0.2 |
| 2018-11 | RAN5#81 | R5-186706 | - | - | - | Added references and common sections | 0.1.0 |
| 2018-11 | RAN5#81 | R5-187847 | - | - | - | Added RMCs, OCGN, SMTC and SSB configurations to Annex A | 0.1.0 |
| 2018-11 | RAN5#81 | R5-187996 | - | - | - | Added test cases 6.7.1.1.1 to 6.7.1.2.2 | 0.1.0 |
| 2018-11 | RAN5#81 | R5-187997 | - | - | - | Added test cases 4.6.2.1 to 4.6.2.8 | 0.1.0 |
| 2018-11 | RAN5#81 | R5-187998 | - | - | - | Added test cases 5.6.2.1 to 5.6.2.4 | 0.1.0 |
| 2018-11 | RAN5#81 | R5-187999 | - | - | - | Added test cases 6.6.2.1 to 6.6.2.8 | 0.1.0 |
| 2018-11 | RAN5#81 | R5-188000 | - | - | - | Added test cases 7.6.2.1 to 7.6.2.4 | 0.1.0 |
| 2018-11 | RAN5#81 | R5-188001 | - | - | - | Added test case 4.4.1.1 | 0.1.0 |
| 2018-11 | RAN5#81 | R5-188002 | - | - | - | Added test cases 4.7.1.1.1 to 4.7.1.2.2 | 0.1.0 |
| 2018-11 | RAN5#81 | R5-188005 | - | - | - | Added Annexes B to H | 0.1.0 |
| 2018-11 | RAN5#81 | R5-188011 | - | - | - | Added test case 4.4.3.1 | 0.1.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190448 | - | - | - | Updating FR1 MU for timing measurements | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190476 | - | - | - | Addition of band group power offsets | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190477 | - | - | - | Update of the annexes | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190478 | - | - | - | Changes to 4.7.1.x tests | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190479 | - | - | - | Addition of 4.7.2.x tests | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190480 | - | - | - | Addition of 4.7.4.x tests | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190481 | - | - | - | Changes to 6.7.1.x tests | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190482 | - | - | - | Addition of 6.7.2.x tests | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190483 | - | - | - | Addition of 6.7.4.x tests | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190512 | - | - | - | Addition of EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode test case 4.5.1.5 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190513 | - | - | - | Addition of EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode test case 4.5.1.6 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190514 | - | - | - | Addition of EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode test case 4.5.1.7 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190515 | - | - | - | Addition of EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode test case 4.5.1.8 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190516 | - | - | - | Addition of EN-DC FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC test case 4.5.2.1 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190517 | - | - | - | Addition of EN-DC FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC test case 4.5.2.2 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190518 | - | - | - | Addition of EN-DC FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC test case 4.5.2.3 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190519 | - | - | - | Addition of EN-DC FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC test case 4.5.2.4 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190520 | - | - | - | Addition of EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC test case 4.5.2.5 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190582 | - | - | - | Annex F correction | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190803 | - | - | - | Update Annex G in TS 38.533 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190874 | - | - | - | Addition of NR test case 6.6.1.1-reporting without gap non-DRX | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190875 | - | - | - | Addition of NR test case 6.6.1.2-reporting without gap DRX | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190876 | - | - | - | Addition of NR test case 6.6.1.3-with gap non DRX | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190877 | - | - | - | Addition of NR test case 6.6.1.4-with gap DRX | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190878 | - | - | - | Addition of NR test case 6.6.1.5-without gap non DRX SBI reading | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190879 | - | - | - | Addition of NR test case 6.6.1.6-with gap non DRX SBI reading | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190880 | - | - | - | CR to 38.533 annex for event triggered reporting test cases | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190881 | - | - | - | Addition of EN-DC FR1 event triggered reporting test case 4.6.1.5 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190882 | - | - | - | Addition of EN-DC FR1 event triggered reporting test case 4.6.1.6 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190883 | - | - | - | Addition of EN-DC FR1 event triggered reporting test case 4.6.1.3 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190884 | - | - | - | Addition of EN-DC FR1 event triggered reporting test case 4.6.1.4 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190885 | - | - | - | Addition of default config for event triggered test cases | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190886 | - | - | - | Introduction of 5G RRM TC 4.5.3.1 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190887 | - | - | - | Introduction of 5G RRM TC 4.5.3.2 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190888 | - | - | - | Introduction of 5G RRM TC 4.5.3.3 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190889 | - | - | - | Introduction of 5G RRM TC 4.6.1.1 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190890 | - | - | - | Introduction of 5G RRM TC 4.6.1.2 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190891 | - | - | - | Introduction of 5G RRM TC 5.5.3.1 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190892 | - | - | - | Introduction of 5G RRM TC 6.6.3.1 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190893 | - | - | - | pCR for Addition of TC 6.5.1.3 NR SA FR1 RLM OOS in DRX | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190894 | - | - | - | pCR for Addition of TC 6.4.3.1 NR SA FR1 TAA Accuracy | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190895 | - | - | - | pCR for Modification of TC 4.4.3.1 EN-DC FR1 TAA accuracy | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190896 | - | - | - | pCR for Addition of TC 6.5.1.1 NR SA FR1 RLM OOS | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190897 | - | - | - | pCR for Addition of TC 4.5.1.1 EN-DC FR1 RLM OOS | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190898 | - | - | - | pCR for Addition of TC 4.5.1.3 EN-DC FR1 RLM OOS in DRX | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190899 | - | - | - | Update to EN-DC FR1 transmit timing accuracy test | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190900 | - | - | - | Addition of EN-DC FR1 RLM IS non-DRX test with SSB-based RLM RS | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190901 | - | - | - | Addition of EN-DC FR1 RLM IS DRX test with SSB-based RLM RS | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190902 | - | - | - | Addition of NR SA FR1 RLM IS non-DRX test with SSB-based RLM RS | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190903 | - | - | - | Addition of NR SA FR1 RLM IS DRX test with SSB-based RLM RS | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190904 | - | - | - | Correction of RRM 5G Test Cases 4.6.2 - EN-DC FR1-FR1 Inter-frequency measurements | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190905 | - | - | - | Correction of RRM 5G Test Cases 6.6.2 - NR SA FR1-FR1 Inter-frequency measurements | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190906 | - | - | - | Correction of RRM 5G Test Cases 7.6.2 - NR SA FR2-FR2 Inter-frequency measurements | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190907 | - | - | - | Addition of RRM Test Cases 4.5.2.6: EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190908 | - | - | - | Addition of RRM Test Cases 5.5.2.1: EN-DC FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190909 | - | - | - | Addition of RRM Test Cases 5.5.2.2: EN-DC FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190910 | - | - | - | Addition of RRM Test Cases 5.5.2.3: EN-DC FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190911 | - | - | - | Addition of RRM Test Cases 5.5.2.4: EN-DC FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190912 | - | - | - | Addition of RRM Test Cases 5.5.2.5: EN-DC FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190913 | - | - | - | Addition of RRM Test Cases 5.5.2.6: EN-DC FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190914 | - | - | - | Addition of RRM Test Cases 6.1.1.1: NR SA FR1 cell re-selection | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190915 | - | - | - | Addition of RRM Test Cases 6.1.1.2: NR SA FR1-FR1 cell re-selection | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190916 | - | - | - | Addition of cell re-selection to higher priority E-UTRAN test case 6.1.2.1 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190917 | - | - | - | Addition of cell re-selection to lower priority E-UTRAN test case 6.1.2.2 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190918 | - | - | - | Addition of SA NR to E-UTRAN handover test case 6.3.1.4 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190919 | - | - | - | Addition of SA NR to E-UTRAN handover test case 6.3.1.5 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190920 | - | - | - | Addition of NR SA FR1 UE UL carrier RRC reconfiguration delay test case 6.5.4.1 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190921 | - | - | - | Addition of NR SA FR1 CSI-RS based RLM out-of-sync non-DRX test case 6.5.1.5 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190922 | - | - | - | Addition of NR SA FR1 CSI-RS based RLM in-sync non-DRX test case 6.5.1.6 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190923 | - | - | - | Addition of NR SA FR1 CSI-RS based RLM out-of-sync in DRX test case 6.5.1.7 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190924 | - | - | - | Addition of NR SA FR1 CSI-RS based RLM in-sync in DRX test case 6.5.1.8 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190987 | - | - | - | 38.533 Common Section updates to clarify leverage across architecture options | 0.2.0 |
| 2019-03 | RAN5#82 | R5-191484 | - | - | - | Correction Annex G | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191485 | - | - | - | Correction NSA Options | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191486 | - | - | - | Modifications NSA FR1 SS-RSRP tests | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191487 | - | - | - | Modifications NSA FR1 SS-RSRQ tests | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191488 | - | - | - | Modifications NSA FR1 L1-RSRP tests | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191489 | - | - | - | Modifications SA FR1 SS-RSRP tests | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191490 | - | - | - | Modifications SA FR1 SS-RSRQ tests | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191491 | - | - | - | Modifications SA FR1 L1-RSRP tests | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191492 | - | - | - | Addition NSA FR1 BWP switch tests | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191493 | - | - | - | Addition SA FR1 BWP switch tests | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191494 | - | - | - | Addition NSA FR2 BWP switch tests | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191495 | - | - | - | Addition SA FR2 BWP switch tests | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191720 | - | - | - | addition of cell mapping for BFD and measurement | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191924 | - | - | - | Correction of default message contents for RRM | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191926 | - | - | - | Addition of event-triggered reporting Test Cases to Cell configuration mapping in Annex E | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191930 | - | - | - | Correction of 5G RRM Test Case 4.6.2.3 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191931 | - | - | - | Correction of 5G RRM Test Case 4.6.2.4 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191934 | - | - | - | Correction of 5G RRM Test Case 4.6.2.7 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191935 | - | - | - | Correction of 5G RRM Test Case 4.6.2.8 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191936 | - | - | - | Addition of Minimum conformance requirements 5.6.2.0 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191937 | - | - | - | Correction of 5G RRM Test Case 5.6.2.1 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191938 | - | - | - | Correction of 5G RRM Test Case 5.6.2.2 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191939 | - | - | - | Correction of 5G RRM Test Case 5.6.2.3 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191940 | - | - | - | Correction of 5G RRM Test Case 5.6.2.4 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191945 | - | - | - | Addition of Minimum conformance requirements 7.6.2.0 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191946 | - | - | - | Correction of 5G RRM Test Case 7.6.2.1 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191947 | - | - | - | Correction of 5G RRM Test Case 7.6.2.2 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191948 | - | - | - | Correction of 5G RRM Test Case 7.6.2.3 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191949 | - | - | - | Correction of 5G RRM Test Case 7.6.2.4 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191950 | - | - | - | Correction of 5G RRM Test Case 7.6.2.5 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191951 | - | - | - | Correction of 5G RRM Test Case 7.6.2.6 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191952 | - | - | - | Correction of 5G RRM Test Case 7.6.2.7 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191953 | - | - | - | Correction of 5G RRM Test Case 7.6.2.8 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192062 | - | - | - | Update of EN-DC FR1 event triggered reporting test case 4.6.1.6 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192063 | - | - | - | Update of EN-DC FR1 event triggered reporting test case 4.6.1.3 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192064 | - | - | - | Update of EN-DC FR1 event triggered reporting test case 4.6.1.4 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192221 | - | - | - | Update on RRC_Connected generic procedure within RRM tests | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192477 | - | - | - | Introduction of FR1 EN-DC Contention based random access Test case | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192478 | - | - | - | Introduction of FR1 EN-DC non-Contention based random access Test case | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192479 | - | - | - | Introduction of FR1 standalone Contention based random access Test case | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192480 | - | - | - | Introduction of FR1 standalone Non-contention based random access Test case | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192481 | - | - | - | Updated to 5G RRM TC 4.6.1.1 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192482 | - | - | - | Updated to 5G RRM TC 4.6.1.2 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192483 | - | - | - | Addition of NR test case 6.7.1.3.1-absolute RSRP | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192484 | - | - | - | Addition of NR test case 6.7.1.3.2-relative RSRP | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192485 | - | - | - | Addition of NR test case 6.5.5.3 FR1 CSI-RS BFD nonDRX | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192486 | - | - | - | Addition of NR test case 6.5.5.4 FR1 CSI-RS BFD DRX | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192487 | - | - | - | Addition of NR test case 7.5.5.1 FR2 SSB BFD nonDRX | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192488 | - | - | - | Addition of NR test case 7.5.5.2 FR2 SSB BFD DRX | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192489 | - | - | - | Addition of NR test case 7.5.5.3 FR2 CSI-RS BFD nonDRX | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192490 | - | - | - | Addition of NR test case 7.5.5.4 FR2 CSI-RS BFD DRX | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192492 | - | - | - | Correction of 5G RRM Test Case 4.6.2.1 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192493 | - | - | - | Correction of 5G RRM Test Case 4.6.2.2 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192494 | - | - | - | Correction of 5G RRM Test Case 4.6.2.5 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192495 | - | - | - | Correction of 5G RRM Test Case 4.6.2.6 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192496 | - | - | - | Correction of 5G RRM Test Case 6.6.2.1 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192497 | - | - | - | Correction of 5G RRM Test Case 6.6.2.2 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192498 | - | - | - | Correction of 5G RRM Test Case 6.6.2.3 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192499 | - | - | - | Correction of 5G RRM Test Case 6.6.2.4 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192500 | - | - | - | Update of EN-DC FR1 event triggered reporting test case 4.6.1.5 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192503 | - | - | - | Update to EN-DC FR1 transmit timing accuracy test | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192674 | - | - | - | Update to EN-DC FR1 RLM IS non-DRX test with SSB-based RLM RS | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192675 | - | - | - | Update to EN-DC FR1 RLM IS DRX test with SSB-based RLM RS | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192676 | - | - | - | Update to NR SA FR1 RLM IS non-DRX test with SSB-based RLM RS | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192677 | - | - | - | Update to NR SA FR1 RLM IS DRX test with SSB-based RLM RS | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192678 | - | - | - | pCR for modification of TAAA TC for EN-DC FR1 | 0.3.0 |
| 2019-03 | RAN#83 | RP-190161 | - | - | - | Presented to the RAN#83 plenary for 1-step approval | 1.0.0 |
| 2019-03 | RAN#83 | - | - | - | - | Upgraded to Rel-15 with small editorial changes | 15.0.0 |
| 2019-06 | RAN#84 | R5-193578 | 0100 | - | F | Update the test case number of TC 4.3.2.2.2 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193755 | 0102 | - | F | Update of Minimum conformance requirements 6.6.1.0 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193758 | 0104 | - | F | Update of EN-DC RLM in-sync in non-DRX test case 4.5.1.6 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193759 | 0105 | - | F | Update of EN-DC RLM out-of-sync in DRX test case 4.5.1.7 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193760 | 0106 | - | F | Update of EN-DC RLM in-sync in DRX test case 4.5.1.8 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193761 | 0107 | - | F | Addition of Minimum conformance requirements 6.3.2 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193762 | 0108 | - | F | Addition of NR RRM TC 6.3.2.1.1-Intra Freq RRC Re-establishment | 15.1.0 |
| 2019-06 | RAN#84 | R5-193763 | 0109 | - | F | Addition of FR1-FR1 re-establishment test case 6.3.2.1.2 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193764 | 0110 | - | F | Addition of FR1 RRC connection release with redirection test case 6.3.2.3.1 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193765 | 0111 | - | F | Addition of FR1-E-UTRA RRC connection release with redirection test case 6.3.2.3.2 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193766 | 0112 | - | F | Update of TC 6.5.1.5 SA FR1 CSI-RS RLM OOS non-DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193767 | 0113 | - | F | Update of TC 6.5.1.6 SA FR1 CSI-RS RLM IS non-DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193768 | 0114 | - | F | Update of TC 6.5.1.7 SA FR1 CSI-RS RLM OOS DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193769 | 0115 | - | F | Update of TC 6.5.1.8 SA FR1 CSI-RS RLM IS DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193770 | 0116 | - | F | Addition of Minimum conformance requirements 6.5.5.0 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193771 | 0117 | - | F | Update of TC 6.5.5.3 SA FR1 CSI-RS BFD non-DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193772 | 0118 | - | F | Update of TC 6.5.5.4 SA FR1 CSI-RS BFD DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193773 | 0119 | - | F | Addition of Minimum conformance requirements 7.5.5.0 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193774 | 0120 | - | F | Update of TC 7.5.5.1 SA FR2 SSB BFD non-DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193775 | 0121 | - | F | Update of TC 7.5.5.2 SA FR2 SSB BFD DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193776 | 0122 | - | F | Update of TC 7.5.5.3 SA FR2 CSI-RS BFD non-DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193777 | 0123 | - | F | Update of TC 7.5.5.4 SA FR2 CSI-RS BFD DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193778 | 0124 | - | F | Addition of Minimum conformance requirements 7.6.1.0 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193779 | 0125 | - | F | Addition of 7.6.1.1 SA FR2 RRM measurement no-gap non-DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193780 | 0126 | - | F | Addition of 7.6.1.2 SA FR2 RRM measurement no-gap DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193781 | 0127 | - | F | Addition of 7.6.1.3 SA FR2 RRM measurement gap non-DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193782 | 0128 | - | F | Addition of 7.6.1.4 SA FR2 RRM measurement gap DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193784 | 0130 | - | F | Update of FR1 cell re-selection test case 6.1.1.1 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193785 | 0131 | - | F | Update of FR1-FR1 cell re-selection test case 6.1.1.2 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193786 | 0132 | - | F | Update of FR1-EUTRA higher priority cell re-selection test case 6.1.2.1 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193787 | 0133 | - | F | Update of FR1-EUTRA lower priority cell re-selection test case 6.1.2.2 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193788 | 0134 | - | F | Update of FR1-EUTRA handover known cell test case 6.3.1.4 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193790 | 0136 | - | F | Update of 6.6.1.1 SA FR1 RRM measurement no-gap non-DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193791 | 0137 | - | F | Update of 6.6.1.2 SA FR1 RRM measurement no-gap DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193792 | 0138 | - | F | Update of 6.6.1.3 SA FR1 RRM measurement gap non-DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193793 | 0139 | - | F | Update of 6.6.1.4 SA FR1 RRM measurement gap DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193794 | 0140 | - | F | Update of FR1 event-triggered without gap with SSB index test case 6.6.1.5 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193795 | 0141 | - | F | Update of FR1 event-triggered with gap with SSB index test case 6.6.1.6 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193812 | 0145 | - | F | Addition of 6.3.2.2 minimum conformance requirements | 15.1.0 |
| 2019-06 | RAN#84 | R5-193872 | 0146 | - | F | Introduction of TC 4.5.5.4 EN-DC FR1 CSI-RS-based beam failure detection and link recovery in DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193949 | 0148 | - | F | Correction of PRACH Configurations | 15.1.0 |
| 2019-06 | RAN#84 | R5-194328 | 0153 | - | F | Additional of new reference used in RRM test spec | 15.1.0 |
| 2019-06 | RAN#84 | R5-194329 | 0154 | - | F | Correction of reference spec number in RRM spec | 15.1.0 |
| 2019-06 | RAN#84 | R5-194494 | 0165 | - | F | Addition missing Editor's note 4.5.2.5 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194549 | 0173 | - | F | Correction of 5G RRM Test Case 5.6.2.1 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194550 | 0174 | - | F | Correction of 5G RRM Test Case 5.6.2.2 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194551 | 0175 | - | F | Correction of 5G RRM Test Case 5.6.2.3 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194552 | 0176 | - | F | Correction of 5G RRM Test Case 5.6.2.4 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194553 | 0177 | - | F | Correction of 5G RRM Test Case 5.6.2.5 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194555 | 0179 | - | F | Correction of 5G RRM Test Case 5.6.2.7 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194556 | 0180 | - | F | Correction of 5G RRM Test Case 5.6.2.8 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194557 | 0181 | - | F | Correction of Minimum conformance requirements 6.6.2.0 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194562 | 0186 | - | F | Correction of Minimum conformance requirements 7.6.2.0 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194564 | 0188 | - | F | Correction of 5G RRM Test Case 7.6.2.2 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194565 | 0189 | - | F | Correction of 5G RRM Test Case 7.6.2.3 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194566 | 0190 | - | F | Correction of 5G RRM Test Case 7.6.2.4 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194567 | 0191 | - | F | Correction of 5G RRM Test Case 7.6.2.5 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194568 | 0192 | - | F | Correction of 5G RRM Test Case 7.6.2.6 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194569 | 0193 | - | F | Correction of 5G RRM Test Case 7.6.2.7 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194570 | 0194 | - | F | Correction of 5G RRM Test Case 7.6.2.8 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194571 | 0195 | - | F | Correction of Minimum conformance requirements 7.3.2.1.0 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194572 | 0196 | - | F | Correction of 5G RRM Test Case 7.3.2.1.1 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194573 | 0197 | - | F | Correction of 5G RRM Test Case 7.3.2.1.2 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194700 | 0208 | - | F | Clean-up in EN-DC FR1 transmit timing accuracy test | 15.1.0 |
| 2019-06 | RAN#84 | R5-194702 | 0210 | - | F | Update to EN-DC FR1 RLM IS DRX test with SSB-based RLM RS | 15.1.0 |
| 2019-06 | RAN#84 | R5-194703 | 0211 | - | F | Update to NR SA FR1 RLM IS non-DRX test with SSB-based RLM RS | 15.1.0 |
| 2019-06 | RAN#84 | R5-194704 | 0212 | - | F | Update to NR SA FR1 RLM IS DRX test with SSB-based RLM RS | 15.1.0 |
| 2019-06 | RAN#84 | R5-194710 | 0214 | - | F | Update to E-UTRA configuration for RRM EN-DC tests to align with core spec update | 15.1.0 |
| 2019-06 | RAN#84 | R5-194713 | 0216 | - | F | Update to EN-DC FR1 RLM IS DRX test with SSB-based RLM RS | 15.1.0 |
| 2019-06 | RAN#84 | R5-195013 | 0156 | 1 | F | Updated to EN-DC FR1 RRM tests in clause 4.5.3 | 15.1.0 |
| 2019-06 | RAN#84 | R5-195016 | 0198 | 1 | F | Test tolerance and measurement uncertainty in Annex F for Inter-Freq measurement test cases | 15.1.0 |
| 2019-06 | RAN#84 | R5-195017 | 0129 | 1 | F | Addition of default configuration in Annex H | 15.1.0 |
| 2019-06 | RAN#84 | R5-195018 | 0143 | 1 | F | Update of 6.3.2.2.1 random access | 15.1.0 |
| 2019-06 | RAN#84 | R5-195019 | 0144 | 1 | F | Update of 6.3.2.2.2 non-contention random access | 15.1.0 |
| 2019-06 | RAN#84 | R5-195020 | 0151 | 1 | F | Updated to EN-DC FR1 RRM tests in clause 4.6.1 | 15.1.0 |
| 2019-06 | RAN#84 | R5-195021 | 0207 | 1 | F | Modification of EN-DC FR1 TAAA | 15.1.0 |
| 2019-06 | RAN#84 | R5-195024 | 0199 | 1 | F | Modification of EN-DC FR1 SSB RLM OOS in non-DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-195025 | 0200 | 1 | F | Modification of EN-DC FR1 SSB RLM OOS in DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-195026 | 0201 | 1 | F | Modification of NR SA FR1 TAAA | 15.1.0 |
| 2019-06 | RAN#84 | R5-195027 | 0202 | 1 | F | Modification of SA FR1 SSB RLM OOS in non-DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-195028 | 0203 | 1 | F | Modification of SA FR1 SSB RLM OOS in DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-195029 | 0204 | 1 | F | Modification of EN-DC FR2 TAAA | 15.1.0 |
| 2019-06 | RAN#84 | R5-195030 | 0205 | 1 | F | Modification of EN-DC FR2 SSB RLM OOS in non-DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-195031 | 0206 | 1 | F | Modification of EN-DC FR2 SSB RLM OOS in DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-195032 | 0219 | 1 | F | Update to EN-DC FR2 RLM IS non-DRX test with SSB-based RLM RS | 15.1.0 |
| 2019-06 | RAN#84 | R5-195033 | 0220 | 1 | F | Update to EN-DC FR2 RLM IS DRX test with SSB-based RLM RS | 15.1.0 |
| 2019-06 | RAN#84 | R5-195034 | 0221 | 1 | F | Addition of EN-DC FR2 Transmit Timing Accuracy tests | 15.1.0 |
| 2019-06 | RAN#84 | R5-195035 | 0209 | 1 | F | Update to EN-DC FR1 RLM IS non-DRX test with SSB-based RLM RS | 15.1.0 |
| 2019-06 | RAN#84 | R5-195036 | 0150 | 1 | F | Correction to EN-DC FR1 UE transmit timing accuracy TC 4.4.1.1 | 15.1.0 |
| 2019-06 | RAN#84 | R5-195037 | 0155 | 1 | F | Updated to cell configuration mapping table for RRM tests | 15.1.0 |
| 2019-06 | RAN#84 | R5-195038 | 0147 | 1 | F | Introduction of TC 6.5.2.1 NR SA FR1 interruptions during measurements on deactivated NR SCC | 15.1.0 |
| 2019-06 | RAN#84 | R5-195039 | 0178 | 1 | F | Correction of 5G RRM Test Case 5.6.2.6 | 15.1.0 |
| 2019-06 | RAN#84 | R5-195040 | 0166 | 1 | F | Correction of default message content for RRM in Annex H | 15.1.0 |
| 2019-06 | RAN#84 | R5-195041 | 0217 | 1 | F | Update to NR SA FR1 RLM IS non-DRX test with SSB-based RLM RS | 15.1.0 |
| 2019-06 | RAN#84 | R5-195042 | 0218 | 1 | F | Update to NR SA FR1 RLM IS DRX test with SSB-based RLM RS | 15.1.0 |
| 2019-06 | RAN#84 | R5-195043 | 0215 | 1 | F | Update to EN-DC FR1 RLM IS non-DRX test with SSB-based RLM RS | 15.1.0 |
| 2019-06 | RAN#84 | R5-195044 | 0187 | 1 | F | Correction of 5G RRM Test Case 7.6.2.1 | 15.1.0 |
| 2019-06 | RAN#84 | R5-195045 | 0103 | 1 | F | Update of EN-DC RLM out-of-sync in non-DRX test case 4.5.1.5 | 15.1.0 |
| 2019-06 | RAN#84 | R5-195099 | 0223 | - | F | RRM implementation of FR2 UL demod OTA tests using single pol Rx TE | 15.1.0 |
| 2019-06 | RAN#84 | R5-195173 | 0167 | 1 | F | Correction of Minimum conformance requirements 4.6.2.0 | 15.1.0 |
| 2019-06 | RAN#84 | R5-195174 | 0168 | 1 | F | Correction of 5G RRM Test Case 4.6.2.1 with FR1 Test tolerance | 15.1.0 |
| 2019-06 | RAN#84 | R5-195175 | 0169 | 1 | F | Correction of 5G RRM Test Case 4.6.2.2 with FR1 Test tolerance | 15.1.0 |
| 2019-06 | RAN#84 | R5-195176 | 0170 | 1 | F | Correction of 5G RRM Test Case 4.6.2.3 with FR1 Test tolerance | 15.1.0 |
| 2019-06 | RAN#84 | R5-195177 | 0171 | 1 | F | Correction of 5G RRM Test Case 4.6.2.4 with FR1 Test tolerance | 15.1.0 |
| 2019-06 | RAN#84 | R5-195179 | 0172 | 1 | F | Correction of Minimum conformance requirements 5.6.2.0 | 15.1.0 |
| 2019-06 | RAN#84 | R5-195182 | 0142 | 1 | F | Addition of FR1-E-UTRAN event-triggered reporting in DRX test case 6.6.3.2 | 15.1.0 |
| 2019-06 | RAN#84 | R5-195185 | 0158 | 1 | F | Update of FR1 Test tolerance and uncertainties in AnnexF | 15.1.0 |
| 2019-06 | RAN#84 | R5-195186 | 0182 | 1 | F | Correction of 5G RRM Test Case 6.6.2.1 with FR1 Test tolerance | 15.1.0 |
| 2019-06 | RAN#84 | R5-195187 | 0183 | 1 | F | Correction of 5G RRM Test Case 6.6.2.2 with FR1 Test tolerance | 15.1.0 |
| 2019-06 | RAN#84 | R5-195188 | 0184 | 1 | F | Correction of 5G RRM Test Case 6.6.2.3 with FR1 Test tolerance | 15.1.0 |
| 2019-06 | RAN#84 | R5-195189 | 0185 | 1 | F | Correction of 5G RRM Test Case 6.6.2.4 with FR1 Test tolerance | 15.1.0 |
| 2019-06 | RAN#84 | R5-195445 | 0152 | 2 | F | Updated to SA FR1 RRM tests in clause 6.6.3 | 15.1.0 |
| 2019-06 | RAN#84 | - | - | - | - | Administrative release upgrade to match the release of 3GPP TS 38.508-1 and TS 38.521-1 which were upgraded at RAN#84 to Rel-16 due to Rel-16 relevant CR(s) | 16.0.0 |
| 2019-09 | RAN#85 | R5-195563 | 0228 | - | F | Addition of 5.6.1.0 minimum requirements | 16.1.0 |
| 2019-09 | RAN#85 | R5-195568 | 0233 | - | F | Update of 6.1.1.2 inter-freq cell re-selection | 16.1.0 |
| 2019-09 | RAN#85 | R5-195569 | 0234 | - | F | Update of 6.1.2.1 inter-RAT cell re-selection to higher priority | 16.1.0 |
| 2019-09 | RAN#85 | R5-195571 | 0236 | - | F | Update of 6.3.1.4 inter-RAT handover to known cell | 16.1.0 |
| 2019-09 | RAN#85 | R5-195573 | 0238 | - | F | Update Test Tolerance of 6.3.2.1.1 intra-freq RRC re-establishment | 16.1.0 |
| 2019-09 | RAN#85 | R5-195574 | 0239 | - | F | Update Test Tolerance of 6.3.2.1.2 inter-freq RRC re-establishment | 16.1.0 |
| 2019-09 | RAN#85 | R5-195575 | 0240 | - | F | Update Test Tolerance of 6.3.2.3.1 NR RRC redirection | 16.1.0 |
| 2019-09 | RAN#85 | R5-195576 | 0241 | - | F | Update Test Tolerance of 6.3.2.3.2 inter-RAT RRC redirection | 16.1.0 |
| 2019-09 | RAN#85 | R5-195577 | 0242 | - | F | Update of 6.5.1.5 RLM out-of-sync non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195578 | 0243 | - | F | Update of 6.5.1.6 RLM in-sync non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195579 | 0244 | - | F | Update of 6.5.1.7 RLM out-of-sync in DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195580 | 0245 | - | F | Update of 6.5.1.8 RLM in-sync in DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195581 | 0246 | - | F | Update of 6.5.5.3 CSI-RS-based BFD non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195582 | 0247 | - | F | Update of 6.5.5.4 CSI-RS-based BFD in DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195596 | 0248 | - | F | Addition of minimum conformance requirements for FR2 EN-DC CSI-RS based RLM | 16.1.0 |
| 2019-09 | RAN#85 | R5-195601 | 0253 | - | F | Addition of minimum conformance requirements for FR2 EN-DC CSI-RS based BFD | 16.1.0 |
| 2019-09 | RAN#85 | R5-195604 | 0256 | - | F | Addition of minimum conformance requirements for SA FR2 reselection | 16.1.0 |
| 2019-09 | RAN#85 | R5-195605 | 0257 | - | F | Addition of NR test case 7.1.1.1-intra freq reselection | 16.1.0 |
| 2019-09 | RAN#85 | R5-195606 | 0258 | - | F | Addition of NR test case 7.1.1.2-inter freq reselection | 16.1.0 |
| 2019-09 | RAN#85 | R5-195608 | 0260 | - | F | Addition of NR test case 8.4.2.1-without SBI non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195609 | 0261 | - | F | Addition of NR test case 8.4.2.2-without SBI DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195610 | 0262 | - | F | Addition of NR test case 8.4.2.3-with SBI non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195611 | 0263 | - | F | Addition of NR test case 8.4.2.4-with SBI DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195612 | 0264 | - | F | Addition of minimum conformance requirements for FR1 EN-DC CSI-RS based RLM | 16.1.0 |
| 2019-09 | RAN#85 | R5-195623 | 0275 | - | F | Correction of NR test case 5.5.2.1-interruption transition DRX sync | 16.1.0 |
| 2019-09 | RAN#85 | R5-195624 | 0276 | - | F | Correction of NR test case 5.5.2.2-interruption transition DRX async | 16.1.0 |
| 2019-09 | RAN#85 | R5-195625 | 0277 | - | F | Correction of NR test case 5.5.2.3-interruption NR deactivated SCell sync | 16.1.0 |
| 2019-09 | RAN#85 | R5-195626 | 0278 | - | F | Correction of NR test case 5.5.2.4-interruption NR deactivated SCell async | 16.1.0 |
| 2019-09 | RAN#85 | R5-195627 | 0279 | - | F | Correction of NR test case 5.5.2.5-interruption LTE deactivated SCell sync | 16.1.0 |
| 2019-09 | RAN#85 | R5-195628 | 0280 | - | F | Correction of NR test case 5.5.2.6-interruption LTE deactivated SCell async | 16.1.0 |
| 2019-09 | RAN#85 | R5-195629 | 0281 | - | F | Correction of NR test case 7.5.5.1-SSB BFD non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195630 | 0282 | - | F | Correction of NR test case 7.5.5.2-SSB BFD DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195631 | 0283 | - | F | Correction of NR test case 7.5.5.3-CSI-RS BFD non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195632 | 0284 | - | F | Correction of NR test case 7.5.5.4-CSI-RS BFD DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195633 | 0285 | - | F | Correction of NR test case 7.6.1.1-without gap non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195634 | 0286 | - | F | Correction of NR test case 7.6.1.2-without gap DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195636 | 0288 | - | F | Correction of NR test case 7.6.1.4-with gap DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-196140 | 0295 | - | F | Update NR operating band groups | 16.1.0 |
| 2019-09 | RAN#85 | R5-196250 | 0296 | - | F | Correction to EN-DC FR1 event-triggered reporting without gap TC 4.6.1.1 and 4.6.1.2 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196503 | 0301 | - | F | Updated to EN-DC FR1 RRM tests in clause 4.5.3 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196504 | 0302 | - | F | Updated to EN-DC FR2 RRM tests in clause 5.5.3 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196547 | 0304 | - | F | Update of FR1 Test tolerance and uncertainties in AnnexF | 16.1.0 |
| 2019-09 | RAN#85 | R5-196601 | 0306 | - | F | Correction of the reference for test frequencies and test mode - Chapter 4 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196602 | 0307 | - | F | Correction of the reference for test frequencies and test mode - Chapter 6 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196603 | 0308 | - | F | Editorial Corrections to section 5.5 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196659 | 0309 | - | F | Editorial corrections of Annex A | 16.1.0 |
| 2019-09 | RAN#85 | R5-196660 | 0310 | - | F | Correction of RRM Test Case 7.3.2.1.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196661 | 0311 | - | F | Correction of RRM Test Case 7.3.2.1.2 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196662 | 0312 | - | F | Correction of cell configuration mapping for RRM Test Cases in Annex E.3 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196666 | 0316 | - | F | Correction of RRM Test Case 5.6.2.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196667 | 0317 | - | F | Correction of RRM Test Case 5.6.2.2 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196668 | 0318 | - | F | Correction of RRM Test Case 5.6.2.3 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196669 | 0319 | - | F | Correction of RRM Test Case 5.6.2.4 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196670 | 0320 | - | F | Correction of RRM Test Case 5.6.2.5 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196671 | 0321 | - | F | Correction of RRM Test Case 5.6.2.6 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196672 | 0322 | - | F | Correction of RRM Test Case 5.6.2.7 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196673 | 0323 | - | F | Correction of RRM Test Case 5.6.2.8 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196674 | 0324 | - | F | Addition of minimum conformance requirements for L1-RSRP measurement RRM FR1 test cases | 16.1.0 |
| 2019-09 | RAN#85 | R5-196675 | 0325 | - | F | Addition of SSB based L1-RSRP measurement RRM test case 4.6.3.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196676 | 0326 | - | F | Addition of SSB based L1-RSRP measurement RRM test case 4.6.3.2 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196677 | 0327 | - | F | Addition of CSI-RS based L1-RSRP measurement RRM test case 4.6.3.3 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196679 | 0329 | - | F | Addition of minimum conformance requirements for L1-RSRP measurement RRM FR2 test cases | 16.1.0 |
| 2019-09 | RAN#85 | R5-196680 | 0330 | - | F | Addition of SSB based L1-RSRP measurement RRM test case 5.6.3.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196681 | 0331 | - | F | Addition of SSB based L1-RSRP measurement RRM test case 5.6.3.2 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196682 | 0332 | - | F | Addition of CSI-RS based L1-RSRP measurement RRM test case 5.6.3.3 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196735 | 0334 | - | F | Editorial to fix conversion issues with two equations | 16.1.0 |
| 2019-09 | RAN#85 | R5-196736 | 0335 | - | F | Adding Chapter 8 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196737 | 0336 | - | F | New test 4.5.7.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196738 | 0337 | - | F | New test 4.5.5.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196739 | 0338 | - | F | New test 4.5.5.2 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196740 | 0339 | - | F | New test 4.7.3.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196741 | 0340 | - | F | New test 4.7.3.2 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196742 | 0341 | - | F | New test 6.7.3.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196743 | 0342 | - | F | New test 6.7.3.2 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196925 | 0351 | - | F | Modification of EN-DC FR2 TAAA Section 5.4.3.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196943 | 0360 | - | F | Addition of CSI-RS based L1-RSRP measurement RRM test case 5.6.3.4 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197361 | 0357 | 1 | F | Modification of SA FR1 TAAA Section 6.4.3.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197364 | 0290 | 1 | F | Update of NR UE Tx Timing Accuracy Test | 16.1.0 |
| 2019-09 | RAN#85 | R5-197366 | 0235 | 1 | F | Update Test Tolerance of 6.1.2.2 inter-RAT cell re-selection to lower priority | 16.1.0 |
| 2019-09 | RAN#85 | R5-197396 | 0346 | 1 | F | Update NSA Event trigger reporting test case | 16.1.0 |
| 2019-09 | RAN#85 | R5-197397 | 0352 | 1 | F | Modification of EN-DC FR2 SSB-RLM OOS Section 5.5.1.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197398 | 0353 | 1 | F | Modification of EN-DC FR2 SSB-RLM OOS in DRX Section 5.5.1.3 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197399 | 0354 | 1 | F | Modification of Synchronous EN-DC FR2 RRC DL BWP Switch in non-DRX Section 5.5.6.2.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197400 | 0355 | 1 | F | Modification of SA FR1 Handover with unknown Target Cell Section 6.3.1.2 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197401 | 0356 | 1 | F | Modification of SA FR1-FR1 Handover with unknown Target Cell Section 6.3.1.3 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197402 | 0224 | 1 | F | Update of 4.6.1.3 event with gap non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197403 | 0225 | 1 | F | Update of 4.6.1.4 event with gap in DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197404 | 0226 | 1 | F | Update of 4.6.1.5 event without gap with ssb index | 16.1.0 |
| 2019-09 | RAN#85 | R5-197405 | 0227 | 1 | F | Update of 4.6.1.6 event with gap with ssb index | 16.1.0 |
| 2019-09 | RAN#85 | R5-197406 | 0314 | 1 | F | Correction of 5G RRM Inter Frequency measurements EN-DC test cases | 16.1.0 |
| 2019-09 | RAN#85 | R5-197407 | 0229 | 1 | F | Addition of 5.6.1.1 event without gap non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197408 | 0230 | 1 | F | Addition of 5.6.1.2 event without gap in DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197409 | 0231 | 1 | F | Addition of 5.6.1.3 event with gap non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197410 | 0232 | 1 | F | Addition of 5.6.1.4 event with gap in DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197411 | 0249 | 1 | F | Addition of NR test case 5.5.1.5-CSI-RS RLM OOS non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197412 | 0250 | 1 | F | Addition of NR test case 5.5.1.6-CSI-RS RLM IS non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197413 | 0251 | 1 | F | Addition of NR test case 5.5.1.7-CSI-RS RLM OOS DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197414 | 0252 | 1 | F | Addition of NR test case 5.5.1.8-CSI-RS RLM IS DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197415 | 0254 | 1 | F | Addition of NR test case 5.5.5.3-CSI-RS BFD non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197416 | 0255 | 1 | F | Addition of NR test case 5.5.5.4-CSI-RS BFD DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197417 | 0259 | 1 | F | Addition of minimum conformance requirements for inter-RAT NR measurements | 16.1.0 |
| 2019-09 | RAN#85 | R5-197418 | 0289 | 1 | F | Correction of default configuration in annex H | 16.1.0 |
| 2019-09 | RAN#85 | R5-197419 | 0303 | 1 | F | Updated to cell configuration mapping table for RRM tests | 16.1.0 |
| 2019-09 | RAN#85 | R5-197420 | 0313 | 1 | F | Correction of default message content for RRM in Annex H | 16.1.0 |
| 2019-09 | RAN#85 | R5-197422 | 0305 | 1 | F | Clarification on NE-DC tests for RRM | 16.1.0 |
| 2019-09 | RAN#85 | R5-197423 | 0343 | 1 | F | Addition RRM FR2 test setups into TS 38.533 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197424 | 0345 | 1 | F | Update TDD UL-DL Config based on TS 38.133 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197425 | 0347 | 1 | F | Modification of EN-DC FR1 UE Transmit Timing Section 4.4.1.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197426 | 0265 | 1 | F | Correction of NR test case 4.5.1.5-CSI-RS RLM OOS non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197427 | 0266 | 1 | F | Correction of NR test case 4.5.1.6-CSI-RS RLM IS non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197428 | 0267 | 1 | F | Correction of NR test case 4.5.1.7-CSI-RS RLM OOS DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197429 | 0268 | 1 | F | Correction of NR test case 4.5.1.8-CSI-RS RLM IS DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197430 | 0269 | 1 | F | Correction of NR test case 4.5.2.1-interruption transition DRX sync | 16.1.0 |
| 2019-09 | RAN#85 | R5-197431 | 0270 | 1 | F | Correction of NR test case 4.5.2.2-interruption transition DRX async | 16.1.0 |
| 2019-09 | RAN#85 | R5-197432 | 0271 | 1 | F | Correction of NR test case 4.5.2.3-interruption NR deactivated SCell sync | 16.1.0 |
| 2019-09 | RAN#85 | R5-197433 | 0272 | 1 | F | Correction of NR test case 4.5.2.4-interruption NR deactivated SCell async | 16.1.0 |
| 2019-09 | RAN#85 | R5-197434 | 0273 | 1 | F | Correction of NR test case 4.5.2.5-interruption LTE deactivated SCell sync | 16.1.0 |
| 2019-09 | RAN#85 | R5-197435 | 0274 | 1 | F | Correction of NR test case 4.5.2.6-interruption LTE deactivated SCell async | 16.1.0 |
| 2019-09 | RAN#85 | R5-197436 | 0328 | 1 | F | Addition of CSI-RS based L1-RSRP measurement RRM test case 4.6.3.4 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197437 | 0315 | 1 | F | Correction of 5G RRM Inter Frequency measurements SA test cases | 16.1.0 |
| 2019-09 | RAN#85 | R5-197570 | 0294 | 1 | F | Annex E and F FR1 Test tolerance update for SS-RSRP | 16.1.0 |
| 2019-09 | RAN#85 | R5-197584 | 0344 | 1 | F | Addition Annex for RRM OTA procedures in TS 38.533 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197607 | 0287 | 1 | F | Correction of NR test case 7.6.1.3-with gap non-DRX. | 16.1.0 |
| 2019-09 | RAN#85 | R5-197611 | 0237 | 1 | F | Update of 6.3.1.5 inter-RAT handover to unknown cell | 16.1.0 |
| 2019-09 | RAN#85 | R5-197612 | 0291 | 1 | F | Update of SSB-based RLM in-sync test cases | 16.1.0 |
| 2019-09 | RAN#85 | R5-197624 | 0297 | 1 | F | Correction to PRACH configurations for FR1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197651 | 0348 | 2 | F | Modification of EN-DC FR1 TAAA Section 4.4.3.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197652 | 0349 | 2 | F | Modification of EN-DC FR1 SSB-RLM OOS Section 4.5.1.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197653 | 0350 | 2 | F | Modification of EN-DC FR1 SSB-RLM OOS DRX Section 4.5.1.3 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197654 | 0358 | 2 | F | Modification of SA FR1 SSB RLM OOS in non-DRX Section 6.5.1.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197655 | 0359 | 2 | F | Modification of SA FR1 SSB RLM OOS in DRX Section 6.5.1.3 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197656 | 0292 | 2 | F | FR1 Test tolerance update for SS-RSRP NSA FR1 test cases | 16.1.0 |
| 2019-09 | RAN#85 | R5-197657 | 0300 | 2 | F | Correction to EN-DC FR1 radio link monitoring TC 4.5.1.1 and 4.5.1.2 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197658 | 0293 | 2 | F | FR1 Test tolerance update for SS-RSRP SA FR1 test cases | 16.1.0 |
| 2019-12 | RAN#86 | R5-197794 | 0369 | - | F | Update Test Tolerance of 4.5.2.1 interruptions active and non-active in sync | 16.2.0 |
| 2019-12 | RAN#86 | R5-197795 | 0370 | - | F | Update Test Tolerance of 4.5.2.2 interruptions active and non-active in async | 16.2.0 |
| 2019-12 | RAN#86 | R5-197804 | 0379 | - | F | Update Test Tolerance of 6.6.3.1 inter-RAT measurement non-DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-197805 | 0380 | - | F | Update Test Tolerance of 6.6.3.2 inter-RAT measurement DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-197813 | 0382 | - | F | Addition of minimum conformance requirements 5.5.5.0.1-SSB based BFD | 16.2.0 |
| 2019-12 | RAN#86 | R5-197814 | 0383 | - | F | Addition of minimum conformance requirements 8.2.1.0-inter-RAT reselection | 16.2.0 |
| 2019-12 | RAN#86 | R5-197815 | 0384 | - | F | Addition of minimum conformance requirements 8.3.1.0-inter-RAT handover | 16.2.0 |
| 2019-12 | RAN#86 | R5-197816 | 0385 | - | F | Addition of minimum conformance requirements 8.4.1.0-inter-RAT SFTD delay | 16.2.0 |
| 2019-12 | RAN#86 | R5-197817 | 0386 | - | F | Addition of minimum conformance requirements 8.5.1.0-inter-RAT SFTD accuracy | 16.2.0 |
| 2019-12 | RAN#86 | R5-197820 | 0389 | - | F | Addition of NR test case 8.2.1.1-high priority NR reselection | 16.2.0 |
| 2019-12 | RAN#86 | R5-198029 | 0394 | - | F | Update NSA SS-RSRP tests for 4Rx connection diagram | 16.2.0 |
| 2019-12 | RAN#86 | R5-198032 | 0397 | - | F | Align Annex D to TS 38.133 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198289 | 0400 | - | F | Correction of PRACH index in 6.1.1.1, 6.3.2.2.1 and 6.3.2.2.2 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198415 | 0404 | - | F | Correction to NR SA FR1 timing advance adjustment accuracy TC 6.4.3.1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198416 | 0405 | - | F | Correction to intra-frequency event-triggered reporting TC 6.6.1.1, 6.6.1.2, 6.6.1.3 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198421 | 0410 | - | F | Correction to FR1 SA random access test cases | 16.2.0 |
| 2019-12 | RAN#86 | R5-198472 | 0412 | - | F | Correct message contents for 5.6.2.x tests | 16.2.0 |
| 2019-12 | RAN#86 | R5-198477 | 0416 | - | F | Correction to Annex E | 16.2.0 |
| 2019-12 | RAN#86 | R5-198518 | 0418 | - | F | Update PUSCH symbol length as message exception in 6.4.1.1 in TS 38.533 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198519 | 0419 | - | F | Update 4.4.3.1 test requirements | 16.2.0 |
| 2019-12 | RAN#86 | R5-198545 | 0420 | - | F | Update 4.7.1.1.1 test requirements in TS 38.533 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198549 | 0421 | - | F | Updated to cell configuration mapping table for RRM tests | 16.2.0 |
| 2019-12 | RAN#86 | R5-198550 | 0422 | - | F | Minimum conformance requirements updated for 5G RRM Inter-RAT measurements tests | 16.2.0 |
| 2019-12 | RAN#86 | R5-198551 | 0423 | - | F | Minimum conformance requirements updated for 5G RRM SCell activation and deactivation delay tests | 16.2.0 |
| 2019-12 | RAN#86 | R5-198558 | 0428 | - | F | 5G RRM Spec 3GPP style correction in clause 4 and 5 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198572 | 0429 | - | F | Correction of minimum conformance requirements 4.6.2.0 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198575 | 0432 | - | F | Correction of minimum conformance requirements 5.6.3.0 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198577 | 0434 | - | F | Correction of clause 3 definitions, symbols and abbreviations | 16.2.0 |
| 2019-12 | RAN#86 | R5-198578 | 0435 | - | F | Correction of RRM Test Case 4.6.3.1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198581 | 0438 | - | F | Correction of RRM Test Case 4.6.3.4 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198582 | 0439 | - | F | Correction of RRM Test Case 5.6.2.1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198583 | 0440 | - | F | Correction of RRM Test Case 5.6.2.2 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198584 | 0441 | - | F | Correction of RRM Test Case 5.6.2.3 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198585 | 0442 | - | F | Correction of RRM Test Case 5.6.2.4 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198586 | 0443 | - | F | Correction of RRM Test Case 5.6.2.5 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198587 | 0444 | - | F | Correction of RRM Test Case 5.6.2.6 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198588 | 0445 | - | F | Correction of RRM Test Case 5.6.2.7 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198589 | 0446 | - | F | Correction of RRM Test Case 5.6.2.8 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198594 | 0451 | - | F | Correction of RRM Test Case 7.6.2.1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198595 | 0452 | - | F | Correction of RRM Test Case 7.6.2.2 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198596 | 0453 | - | F | Correction of RRM Test Case 7.6.2.3 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198597 | 0454 | - | F | Correction of RRM Test Case 7.6.2.4 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198598 | 0455 | - | F | Correction of RRM Test Case 7.6.2.5 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198599 | 0456 | - | F | Correction of RRM Test Case 7.6.2.6 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198601 | 0458 | - | F | Correction of RRM Test Case 7.6.2.8 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198692 | 0471 | - | F | Addition of RRM Test Case 6.6.4.1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198693 | 0472 | - | F | Addition of RRM Test Case 6.6.4.2 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198694 | 0473 | - | F | Addition of RRM Test Case 6.6.4.3 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198695 | 0474 | - | F | Addition of RRM Test Case 6.6.4.4 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198696 | 0475 | - | F | Addition of RRM Test Case 7.6.3.1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198697 | 0476 | - | F | Addition of RRM Test Case 7.6.3.2 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198698 | 0477 | - | F | Addition of RRM Test Case 7.6.3.3 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198699 | 0478 | - | F | Addition of RRM Test Case 7.6.3.4 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199364 | 0365 | 1 | F | Update Test Tolerance of 4.5.1.5 RLM OOS non-DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-199365 | 0367 | 1 | F | Update Test Tolerance of 4.5.1.7 RLM OOS DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-199366 | 0375 | 1 | F | Update Test Tolerance of 6.5.1.5 SA RLM OOS non-DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-199367 | 0377 | 1 | F | Update Test Tolerance of 6.5.1.7 SA RLM OOS DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-199368 | 0366 | 1 | F | Update Test Tolerance of 4.5.1.6 RLM in-sync non-DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-199369 | 0368 | 1 | F | Update Test Tolerance of 4.5.1.8 RLM in-sync DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-199370 | 0376 | 1 | F | Update Test Tolerance of 6.5.1.6 SA RLM in-sync non-DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-199371 | 0378 | 1 | F | Update Test Tolerance of 6.5.1.8 SA RLM in-sync DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-199379 | 0417 | 1 | F | Update 4.4.1.1 test procedure in TS 38.533 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199380 | 0479 | 1 | F | Update 4.5.1.1 and 4.5.1.2 in TS 38.533 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199381 | 0406 | 1 | F | Correction to NR SA FR1 SS-RSRP absolute and relative measurement accuracy Intra-frequency measurements | 16.2.0 |
| 2019-12 | RAN#86 | R5-199389 | 0402 | 1 | F | Correction to NR SA Cell re-selection tests | 16.2.0 |
| 2019-12 | RAN#86 | R5-199390 | 0436 | 1 | F | Correction of RRM Test Case 4.6.3.2 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199391 | 0437 | 1 | F | Correction of RRM Test Case 4.6.3.3 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199392 | 0387 | 1 | F | Addition of NR test case 5.5.5.1-ENDC FR2 SSB BFD no-DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-199393 | 0388 | 1 | F | Addition of NR test case 5.5.5.2-ENDC FR2 SSB BFD DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-199394 | 0431 | 1 | F | Correction of minimum conformance requirements 5.6.2.0 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199395 | 0395 | 1 | F | Update SA SS-RSRP tests for 4Rx connection diagram | 16.2.0 |
| 2019-12 | RAN#86 | R5-199396 | 0403 | 1 | F | Correction to NR SA FR1 UE transmit timing accuracy TC 6.4.1.1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199397 | 0457 | 1 | F | Correction of RRM Test Case 7.6.2.7 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199398 | 0361 | 1 | F | Add AoA Setup 4 for FR2 RRM Test cases | 16.2.0 |
| 2019-12 | RAN#86 | R5-199399 | 0390 | 1 | F | Addition of NR test case 8.3.1.1-known handover | 16.2.0 |
| 2019-12 | RAN#86 | R5-199400 | 0391 | 1 | F | Addition of NR test case 8.4.1.1-SFTD delay non-DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-199401 | 0392 | 1 | F | Addition of NR test case 8.4.1.2-SFTD delay DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-199402 | 0393 | 1 | F | Addition of NR test case 8.5.1.1-SFTD accuracy | 16.2.0 |
| 2019-12 | RAN#86 | R5-199403 | 0396 | 1 | F | Align Annex A to TS 38.133 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199404 | 0413 | 1 | F | Clean up RRM message contents in Annex H | 16.2.0 |
| 2019-12 | RAN#86 | R5-199405 | 0424 | 1 | F | New 5G RRM Inter-RAT measurement TC 8.4.2.5 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199406 | 0425 | 1 | F | New 5G RRM Inter-RAT measurement TC 8.4.2.6 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199407 | 0426 | 1 | F | New 5G RRM Inter-RAT measurement TC 8.4.2.7 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199408 | 0427 | 1 | F | New 5G RRM Inter-RAT measurement TC 8.4.2.8 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199409 | 0433 | 1 | F | Correction of message content in Annex H | 16.2.0 |
| 2019-12 | RAN#86 | R5-199436 | 0470 | 1 | F | Introduction of n29 and n65 to 38.533 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199499 | 0401 | 1 | F | Correction to EN-DC FR1 UE transmit timing accuracy TC 4.4.1.1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199515 | 0381 | 1 | F | Update Test Tolerance in Annex F | 16.2.0 |
| 2019-12 | RAN#86 | R5-199517 | 0481 | 1 | F | Update General parameters in test case 4.5.1.2 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199533 | 0482 | 1 | F | Update NSA FR2 RLM IS non-DRX test | 16.2.0 |
| 2019-12 | RAN#86 | R5-199534 | 0483 | 1 | F | Update NSA FR2 RLM IS test with DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-199535 | 0484 | 1 | F | Update NSA FR2 Timing Accuracy Test | 16.2.0 |
| 2019-12 | RAN#86 | R5-199544 | 0399 | 1 | F | Add applicable test methods for RRM FR2 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199550 | 0463 | 1 | F | Addition of NR SA FR1 Handover with known Target Cell Section 6.3.1.1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199551 | 0464 | 1 | F | Modification of NR SA FR1 Handover with unknown Target Cell Section 6.3.1.2 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199552 | 0465 | 1 | F | Modification of NR SA FR1-FR1 Handover with unknown Target Cell Section 6.3.1.3 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199583 | 0409 | 1 | F | Correction to FR1 NSA random access test cases | 16.2.0 |
| 2020-03 | RAN#87 | R5-200822 | 0544 | - | F | Correction to Test Applicability for 4.3.2.2.2 Non-contention based random access test in FR1 for PSCell in EN-DC | 16.3.0 |
| 2020-03 | RAN#87 | R5-200823 | 0545 | - | F | Correction to SRS Configuration for 4.4.3.1 EN-DC FR1 timing advance adjustment accuracy | 16.3.0 |
| 2020-03 | RAN#87 | R5-200829 | 0551 | - | F | Correction to Message Exception for 4.6.1.5 EN-DC FR1 event-triggered reporting without gap in non-DRX with SSB time index detection | 16.3.0 |
| 2020-03 | RAN#87 | R5-200833 | 0555 | - | F | Correction to Test Applicability for 6.3.2.2.2 Non-Contention based random access test in FR1 for NR standalone | 16.3.0 |
| 2020-03 | RAN#87 | R5-200834 | 0556 | - | F | Correction to SRS Configuration for 6.4.3.1 NR SA FR1 timing advance adjustment accuracy | 16.3.0 |
| 2020-03 | RAN#87 | R5-200839 | 0561 | - | F | Correction to Message Exception for 6.6.1.5 NR SA FR1 event-triggered reporting without gap in non-DRX with SSB time index detection | 16.3.0 |
| 2020-03 | RAN#87 | R5-200277 | 0488 | - | F | Test tolerance update intra-frequency SS-SINR NSA | 16.3.0 |
| 2020-03 | RAN#87 | R5-200278 | 0489 | - | F | Test tolerance update inter-frequency SS-SINR NSA | 16.3.0 |
| 2020-03 | RAN#87 | R5-200279 | 0490 | - | F | Test tolerance update inter-frequency SS-RSRP SA | 16.3.0 |
| 2020-03 | RAN#87 | R5-200281 | 0492 | - | F | Test tolerance update inter-frequency SS-RSRQ SA | 16.3.0 |
| 2020-03 | RAN#87 | R5-200282 | 0493 | - | F | Test tolerance update intra-frequency SS-SINR SA | 16.3.0 |
| 2020-03 | RAN#87 | R5-200283 | 0494 | - | F | Test tolerance update inter-frequency SS-SINR SA | 16.3.0 |
| 2020-03 | RAN#87 | R5-200306 | 0497 | - | F | Reference SSB configuration correction | 16.3.0 |
| 2020-03 | RAN#87 | R5-200421 | 0508 | - | F | Correction to Active UL BWP for intra-frequency event triggered reporting with gap | 16.3.0 |
| 2020-03 | RAN#87 | R5-200422 | 0509 | - | F | Correction to EN-DC FR1 event-triggered reporting test cases | 16.3.0 |
| 2020-03 | RAN#87 | R5-200423 | 0510 | - | F | Correction to FR1 NR SA E-UTRA cell re-selection test cases | 16.3.0 |
| 2020-03 | RAN#87 | R5-200425 | 0512 | - | F | Correction to NR SA FR1 SS-RSRP measurement accuracy tests | 16.3.0 |
| 2020-03 | RAN#87 | R5-200427 | 0514 | - | F | Correction to the simulated cell for cell-reselection test cases | 16.3.0 |
| 2020-03 | RAN#87 | R5-200428 | 0515 | - | F | Correction to UE transmit timing accuracy test case | 16.3.0 |
| 2020-03 | RAN#87 | R5-200564 | 0524 | - | F | Correction to Statistical testing of delay and UE measurement performance in RRM tests | 16.3.0 |
| 2020-03 | RAN#87 | R5-200606 | 0526 | - | F | Cell mapping update measurement tests Annex E | 16.3.0 |
| 2020-03 | RAN#87 | R5-200607 | 0527 | - | F | Re-submission R5-197804 not implemented | 16.3.0 |
| 2020-03 | RAN#87 | R5-200608 | 0528 | - | F | Corrections to SS-RSRP meas accuracy NSA tests | 16.3.0 |
| 2020-03 | RAN#87 | R5-200610 | 0530 | - | F | Correct message contents measurement tests | 16.3.0 |
| 2020-03 | RAN#87 | R5-200705 | 0533 | - | F | Correction to TC 6.1.1.2 FR1 inter-freq re-selection | 16.3.0 |
| 2020-03 | RAN#87 | R5-200708 | 0536 | - | F | Update of TC 6.3.2.3.1 FR1 RRC redirection | 16.3.0 |
| 2020-03 | RAN#87 | R5-200709 | 0537 | - | F | Update of TC 6.3.2.3.2 inter-freq RRC redirection | 16.3.0 |
| 2020-03 | RAN#87 | R5-200785 | 0539 | - | F | Update of EN-DC SSB-based RLM TC 4.5.1.1, 4.5.1.2, 4.5.1.3 and 4.5.1.4 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200786 | 0540 | - | F | Update of EN-DC CSI-RS-based RLM TC 4.5.1.5, 4.5.1.6, 4.5.1.7 and 4.5.1.8 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200787 | 0541 | - | F | Update of NR CSI-RS-based RLM TC 6.5.1.5, 6.5.1.6, 6.5.1.7 and 6.5.1.8 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200818 | 0543 | - | F | Correction to cell mapping Annex E | 16.3.0 |
| 2020-03 | RAN#87 | R5-200858 | 0569 | - | F | Corrections to Table H.3.1-8 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200916 | 0538 | 1 | F | Update to test applicability per permitted test method | 16.3.0 |
| 2020-03 | RAN#87 | R5-200986 | 0507 | 1 | F | Core alignment for event-triggered reporting test cases | 16.3.0 |
| 2020-03 | RAN#87 | R5-200987 | 0534 | 1 | F | Update of TC 6.3.2.1.1 FR1 RRC re-establishment | 16.3.0 |
| 2020-03 | RAN#87 | R5-200988 | 0535 | 1 | F | Update of TC 6.3.2.1.2 inter-freq RRC re-establishment | 16.3.0 |
| 2020-03 | RAN#87 | R5-201030 | 0529 | 1 | F | Corrections to SS-RSRP meas accuracy SA tests | 16.3.0 |
| 2020-03 | RAN#87 | R5-201039 | 0485 | 1 | F | Test tolerance update inter-frequency SS-RSRP NSA | 16.3.0 |
| 2020-03 | RAN#87 | R5-201040 | 0486 | 1 | F | Test tolerance update intra-frequency SS-RSRQ NSA | 16.3.0 |
| 2020-03 | RAN#87 | R5-201041 | 0491 | 1 | F | Test tolerance update intra-frequency SS-RSRQ SA | 16.3.0 |
| 2020-03 | RAN#87 | R5-201056 | 0511 | 1 | F | Correction to message contents in 6.6.1.5 and 6.6.1.6 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201057 | 0513 | 1 | F | Correction to test description of RRM TC 4.6.1.3 and 4.6.1.6 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201058 | 0516 | 1 | F | Update of maximum test system uncertainty for FR1 RRM Test | 16.3.0 |
| 2020-03 | RAN#87 | R5-201063 | 0517 | 1 | F | Update FR1 Test Tolerance of 4.5.2.3 interruptions on NR SCC in sync | 16.3.0 |
| 2020-03 | RAN#87 | R5-201064 | 0518 | 1 | F | Update FR1 Test Tolerance of 4.5.2.4 interruptions on NR SCC in async | 16.3.0 |
| 2020-03 | RAN#87 | R5-201071 | 0552 | 1 | F | Correction to Test Applicability for 4.6.2.2 EN-DC FR1-FR1 event-triggered reporting in DRX | 16.3.0 |
| 2020-03 | RAN#87 | R5-201072 | 0553 | 1 | F | Correction to Test Applicability for 4.6.2.5 EN-DC FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | 16.3.0 |
| 2020-03 | RAN#87 | R5-201073 | 0554 | 1 | F | Correction to Test Applicability for 4.6.2.6 EN-DC FR1-FR1 event-triggered reporting in DRX with SSB time index detection | 16.3.0 |
| 2020-03 | RAN#87 | R5-201078 | 0562 | 1 | F | Correction to Test Applicability for 6.6.2.1 NR SA FR1-FR1 event-triggered reporting in non-DRX | 16.3.0 |
| 2020-03 | RAN#87 | R5-201080 | 0564 | 1 | F | Correction to Test Applicability for 6.6.2.5 NR SA FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | 16.3.0 |
| 2020-03 | RAN#87 | R5-201081 | 0565 | 1 | F | Correction to Test Applicability for 6.6.2.6 NR SA FR1-FR1 event-triggered reporting in DRX with SSB time index detection | 16.3.0 |
| 2020-03 | RAN#87 | R5-201082 | 0571 | 1 | F | Correction to Test Parameters for 4.6.1.1 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201083 | 0572 | 1 | F | Correction to Test Parameters for 4.6.1.2 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201160 | 0531 | 1 | F | Correct message contents event triggered measurement tests | 16.3.0 |
| 2020-03 | RAN#87 | R5-201166 | 0567 | 1 | F | Update RLM non DRX test cases | 16.3.0 |
| 2020-03 | RAN#87 | R5-201167 | 0568 | 1 | F | Update 4.5.1.4 test case | 16.3.0 |
| 2020-03 | RAN#87 | R5-201168 | 0570 | 1 | F | Clarification 4.4.1.1 test procedure | 16.3.0 |
| 2020-03 | RAN#87 | R5-201169 | 0521 | 1 | F | Update of FR1 Test Tolerance in Annex F | 16.3.0 |
| 2020-03 | RAN#87 | R5-201190 | 0542 | 1 | F | Update of NR SSB-based RLM TC 6.5.1.1, 6.5.1.2, 6.5.1.3 and 6.5.1.4 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201240 | 0563 | 2 | F | Correction to Test Applicability for 6.6.2.2 NA SA FR1-FR1 event-triggered reporting in DRX | 16.3.0 |
| 2020-03 | RAN#87 | R5-201242 | 0532 | 2 | F | Correction to Test Applicability for 4.6.2.1 EN-DC FR1-FR1 event-triggered reporting in non-DRX | 16.3.0 |
| 2020-03 | RAN#87 | R5-200276 | 0487 | - | F | Test tolerance update inter-frequency SS-RSRQ NSA | 16.3.0 |
| 2020-03 | RAN#87 | R5-200284 | 0495 | - | F | Test tolerance update measurement tests Annex F | 16.3.0 |
| 2020-03 | RAN#87 | R5-201044 | 0496 | - | F | Test Tolerance and Measurement Uncertainty in Annex F for L1-RSRP measurement test cases | 16.3.0 |
| 2020-03 | RAN#87 | R5-201045 | 0499 | 1 | F | Correction of RRM Test Case 4.6.3.1 including FR1 Test Tolerance | 16.3.0 |
| 2020-03 | RAN#87 | R5-201046 | 0500 | 1 | F | Correction of RRM Test Case 4.6.3.2 including FR1 Test Tolerance | 16.3.0 |
| 2020-03 | RAN#87 | R5-201047 | 0501 | 1 | F | Correction of RRM Test Case 4.6.3.3 including FR1 Test Tolerance | 16.3.0 |
| 2020-03 | RAN#87 | R5-201048 | 0502 | 1 | F | Correction of RRM Test Case 4.6.3.4 including FR1 Test Tolerance | 16.3.0 |
| 2020-03 | RAN#87 | R5-201049 | 0503 | 1 | F | Correction of RRM Test Case 6.6.4.1 including FR1 Test Tolerance | 16.3.0 |
| 2020-03 | RAN#87 | R5-201050 | 0504 | 1 | F | Correction of RRM Test Case 6.6.4.2 including FR1 Test Tolerance | 16.3.0 |
| 2020-03 | RAN#87 | R5-201051 | 0505 | 1 | F | Correction of RRM Test Case 6.6.4.3 including FR1 Test Tolerance | 16.3.0 |
| 2020-03 | RAN#87 | R5-201052 | 0506 | 1 | F | Correction of RRM Test Case 6.6.4.4 including FR1 Test Tolerance | 16.3.0 |
| 2020-03 | RAN#87 | R5-201070 | 0548 | 1 | F | Correction to Message Exception for 4.5.1.3 EN-DC FR1 radio link monitoring Out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode | 16.3.0 |
| 2020-03 | RAN#87 | R5-201074 | 0557 | 1 | F | Correction to Test Parameter for 6.5.1.1 NR SA FR1 radio link monitoring Out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | 16.3.0 |
| 2020-03 | RAN#87 | R5-201075 | 0558 | 1 | F | Correction to Test Procedure for 6.5.1.2 NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode | 16.3.0 |
| 2020-03 | RAN#87 | R5-201076 | 0559 | 1 | F | Correction to Message Exception for 6.5.1.3 NR SA FR1 radio link monitoring Out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode | 16.3.0 |
| 2020-03 | RAN#87 | R5-201077 | 0560 | 1 | F | Correction to Message Exception for 6.5.1.4 NR SA FR1 radio link monitoring In-sync test for PSCell configured with SSB-based RLM RS in DRX mode | 16.3.0 |
| 2020-03 | RAN#87 | R5-201105 | 0546 | 1 | F | Correction to Test Parameters for 4.5.1.1 EN-DC FR1 radio link monitoring Out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | 16.3.0 |
| 2020-03 | RAN#87 | R5-201106 | 0547 | 1 | F | Correction to Test Procedure for 4.5.1.2 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | 16.3.0 |
| 2020-06 | RAN#88 | R5-201395 | 0576 | - | F | Correction to NR test case 7.7.1.3.2 SS-RSRP relative accuracy | 16.4.0 |
| 2020-06 | RAN#88 | R5-201396 | 0577 | - | F | Correction to NR test case 8.2.1.1 higher priority NR cell reselection | 16.4.0 |
| 2020-06 | RAN#88 | R5-201398 | 0579 | - | F | Correction to NR test case 8.4.1.1 SFTD non-DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201399 | 0580 | - | F | Correction to NR test case 8.4.1.2 SFTD DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201400 | 0581 | - | F | Correction to NR test case 8.4.2.1 event-triggered without SSB index non-DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201401 | 0582 | - | F | Correction to NR test case 8.4.2.2 event-triggered without SSB index DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201402 | 0583 | - | F | Correction to NR test case 8.4.2.3 event-triggered with SSB index non-DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201403 | 0584 | - | F | Correction to NR test case 8.4.2.4 event-triggered with SSB index DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201404 | 0585 | - | F | Correction to NR test case 8.5.1.1 SFTD accuracy | 16.4.0 |
| 2020-06 | RAN#88 | R5-201405 | 0586 | - | F | Addition to minimum conformance requirement 5.5.5.0.3 - scheduling restriction for BFD and CBD | 16.4.0 |
| 2020-06 | RAN#88 | R5-201407 | 0588 | - | F | Addition to minimum conformance requirement 5.5.8.0 - TCI-state switch | 16.4.0 |
| 2020-06 | RAN#88 | R5-201408 | 0589 | - | F | Addition to NR test case 5.5.8.1 MAC-CE based TCI switch | 16.4.0 |
| 2020-06 | RAN#88 | R5-201409 | 0590 | - | F | Addition to NR test case 5.5.8.2 RRC based TCI switch | 16.4.0 |
| 2020-06 | RAN#88 | R5-201412 | 0593 | - | F | Addition to minimum conformance requirements 7.5.1.0.5 - Scheduling restriction for RLM | 16.4.0 |
| 2020-06 | RAN#88 | R5-201414 | 0595 | - | F | Addition to minimum Conformance Requirements 7.5.6.1.0 - DCI-based DL active BWP switch | 16.4.0 |
| 2020-06 | RAN#88 | R5-201415 | 0596 | - | F | Addition to NR test case 7.5.6.1.3 FR2 DCI-based DL active BWP switch non-DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201619 | 0603 | - | F | Corrections to 6.7.1.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201676 | 0614 | - | F | Correction to inter-freq measurement TCs for SSB configuration | 16.4.0 |
| 2020-06 | RAN#88 | R5-201677 | 0615 | - | F | Correction to TC 6.3.2.3.1 for the test procedure | 16.4.0 |
| 2020-06 | RAN#88 | R5-201679 | 0617 | - | F | Update of TC 5.6.1.1 event-triggered without gap non-DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201680 | 0618 | - | F | Update of TC 5.6.1.2 event-triggered without gap DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201681 | 0619 | - | F | Update of TC 5.6.1.3 event-triggered with gap non-DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201682 | 0620 | - | F | Update of TC 5.6.1.4 event-triggered with gap DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201686 | 0624 | - | F | Update of TC 8.4.2.5 FR2 event-triggered without SSB index non-DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201687 | 0625 | - | F | Update of TC 8.4.2.6 FR2 event-triggered without SSB index DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201688 | 0626 | - | F | Update of TC 8.4.2.7 FR2 event-triggered with SSB index non-DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201689 | 0627 | - | F | Update of TC 8.4.2.8 FR2 event-triggered with SSB index DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201821 | 0634 | - | F | Correction of SS-SINR applicability -NSA | 16.4.0 |
| 2020-06 | RAN#88 | R5-201822 | 0635 | - | F | Correction of SS-SINR applicability -SA | 16.4.0 |
| 2020-06 | RAN#88 | R5-201824 | 0637 | - | F | Corrections to 4.7.1.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202054 | 0644 | - | F | Minimum Requirements SS-RSRP iRAT | 16.4.0 |
| 2020-06 | RAN#88 | R5-202055 | 0645 | - | F | Addition 8.5.2.1.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202056 | 0646 | - | F | Addition 8.5.2.1.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202057 | 0647 | - | F | Minimum Requirements SS-RSRQ iRAT | 16.4.0 |
| 2020-06 | RAN#88 | R5-202058 | 0648 | - | F | Addition 8.5.2.2.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202059 | 0649 | - | F | Addition 8.5.2.2.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202060 | 0650 | - | F | Minimum Requirements SS-SINR iRAT | 16.4.0 |
| 2020-06 | RAN#88 | R5-202061 | 0651 | - | F | Addition 8.5.2.3.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202062 | 0652 | - | F | Addition 8.5.2.3.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202063 | 0653 | - | F | Minimum requirements SS-RSRP NSA FR2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202064 | 0654 | - | F | Addition 5.7.1.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202065 | 0655 | - | F | Addition 5.7.1.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202066 | 0656 | - | F | Addition 5.7.1.3 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202067 | 0657 | - | F | Minimum requirements SS-RSRQ NSA FR2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202068 | 0658 | - | F | Addition 5.7.2.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202069 | 0659 | - | F | Addition 5.7.2.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202070 | 0660 | - | F | Minimum requirements SS-SINR NSA FR2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202071 | 0661 | - | F | Addition 5.7.3.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202072 | 0662 | - | F | Addition 5.7.3.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202073 | 0663 | - | F | Minimum requirements SS-RSRP SA FR2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202074 | 0664 | - | F | Addition 7.7.1.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202075 | 0665 | - | F | Addition 7.7.1.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202076 | 0666 | - | F | Minimum requirements SS-RSRQ SA FR2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202077 | 0667 | - | F | Addition 7.7.2.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202078 | 0668 | - | F | Addition 7.7.2.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202079 | 0669 | - | F | Minimum requirements SS-SINR SA FR2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202080 | 0670 | - | F | Addition 7.7.3.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202081 | 0671 | - | F | Addition 7.7.3.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202091 | 0672 | - | F | Correction of OCNG configurations and CSI-RS for tracking in Annex A | 16.4.0 |
| 2020-06 | RAN#88 | R5-202092 | 0673 | - | F | Cell configuration for SA FR2 event-triggered reporting test cases | 16.4.0 |
| 2020-06 | RAN#88 | R5-202093 | 0674 | - | F | Correction of MeasGapConfig in Annex H | 16.4.0 |
| 2020-06 | RAN#88 | R5-202095 | 0676 | - | F | Correction of AoA configuration for FR2 SA Inter-freq measurement test cases in clause 5.6.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202096 | 0677 | - | F | Correction of RRM Test Case 7.6.2.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202097 | 0678 | - | F | Correction of RRM Test Case 7.6.2.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202098 | 0679 | - | F | Correction of RRM Test Case 7.6.2.3 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202099 | 0680 | - | F | Correction of RRM Test Case 7.6.2.4 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202100 | 0681 | - | F | Correction of RRM Test Case 7.6.2.5 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202101 | 0682 | - | F | Correction of RRM Test Case 7.6.2.6 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202102 | 0683 | - | F | Correction of RRM Test Case 7.6.2.7 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202103 | 0684 | - | F | Correction of RRM Test Case 7.6.2.8 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202263 | 0704 | - | F | Core alignment in 4.6.1.5 and 4.6.1.6 Event Triggered Reporting with SSB time index detection | 16.4.0 |
| 2020-06 | RAN#88 | R5-202265 | 0706 | - | F | Correction on EN-DC FR1 radio link monitoring out-of-sync test | 16.4.0 |
| 2020-06 | RAN#88 | R5-202273 | 0714 | - | F | Correction to NR SA FR1 - E-UTRA RRC Connection release with redirection | 16.4.0 |
| 2020-06 | RAN#88 | R5-202277 | 0718 | - | F | Editorial Correction to FR2 Interruption test cases | 16.4.0 |
| 2020-06 | RAN#88 | R5-202385 | 0720 | - | F | Addition of RRM Cell configuration mapping table for EN-DC FR1 Test Case 4.5.4.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202387 | 0721 | - | F | Addition of RRC message content exceptions for UE UL Carrier Reconfiguration Delay | 16.4.0 |
| 2020-06 | RAN#88 | R5-202389 | 0722 | - | F | Addition of RRM Test Case 4.5.4.1 in Annex F | 16.4.0 |
| 2020-06 | RAN#88 | R5-202415 | 0728 | - | F | Alignment 5.4.1.1 test procedure with 4.4.1.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202418 | 0730 | - | F | Update 6.5.1.1 and CSI-ReportConfig in Annex H | 16.4.0 |
| 2020-06 | RAN#88 | R5-202488 | 0736 | - | F | Correction to the simulated cell for cell-reselection test cases | 16.4.0 |
| 2020-06 | RAN#88 | R5-202496 | 0744 | - | F | Correction of RRM Test Case 6.4.1.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202497 | 0745 | - | F | Title Correction of Annex D.4.2.4 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202700 | 0723 | 1 | F | Addition of new RRM Test Case 4.5.4.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202701 | 0675 | 1 | F | Editorial correction of RRM TT in Annex F | 16.4.0 |
| 2020-06 | RAN#88 | R5-202744 | 0708 | 1 | F | Correction to EN-DC FR1 radio link monitoring tests | 16.4.0 |
| 2020-06 | RAN#88 | R5-202745 | 0727 | 1 | F | Clarification cell2 SS-RSRP in 4.7.1.2.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202746 | 0587 | 1 | F | Addition to NR test case 5.5.5.5 scheduling available restriction | 16.4.0 |
| 2020-06 | RAN#88 | R5-202747 | 0707 | 1 | F | Correction on NR SA FR1 - E-UTRAN event-triggered reporting tests | 16.4.0 |
| 2020-06 | RAN#88 | R5-202748 | 0731 | 1 | F | Update 6.5.1.2 message content exception and test procedure | 16.4.0 |
| 2020-06 | RAN#88 | R5-202749 | 0737 | 1 | F | Correction of RRM Test Case 6.1.1.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202750 | 0738 | 1 | F | Correction of RRM Test Case 6.1.1.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202751 | 0621 | 1 | F | Update of TC 7.1.1.1 intra-freq cell re-selection | 16.4.0 |
| 2020-06 | RAN#88 | R5-202752 | 0622 | 1 | F | Update of TC 7.1.1.2 inter-freq cell re-selection | 16.4.0 |
| 2020-06 | RAN#88 | R5-202753 | 0724 | 1 | F | Addition of new RRM Test Cases in clause 7.5.3 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202754 | 0725 | 1 | F | Addition of new RRM Test Cases in clause 7.5.7 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202812 | 0685 | 1 | F | Correction to Test Applicability for 4.6.1.3 EN-DC FR1 event-triggered reporting with gap in non-DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-202813 | 0686 | 1 | F | Correction to Test Applicability for 4.6.1.4 EN-DC FR1 event-triggered reporting with gap in DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-202814 | 0687 | 1 | F | Correction to Test Applicability & Message Exception for 4.6.1.6 EN-DC FR1 event-triggered reporting with gap in non-DRX with SSB time index detection | 16.4.0 |
| 2020-06 | RAN#88 | R5-202815 | 0688 | 1 | F | Correction to Test Applicability for 6.6.1.3 NR SA FR1 event-triggered reporting with gap in non-DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-202816 | 0689 | 1 | F | Correction to Test Applicability for 6.6.1.4 NR SA FR1 event-triggered reporting with gap in DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-202817 | 0732 | 1 | F | Correction to Test Requirement for 4.4.3.1 by updating TT values from Annex F | 16.4.0 |
| 2020-06 | RAN#88 | R5-202833 | 0591 | 1 | F | Addition to NR test case 6.3.2.1.3 FR1-FR1 RRC re-establishment without serving cell timing | 16.4.0 |
| 2020-06 | RAN#88 | R5-202834 | 0575 | 1 | F | Correction to NR test case 7.7.1.3.1 SA FR2 SS-RSRP absolute accuracy | 16.4.0 |
| 2020-06 | RAN#88 | R5-202835 | 0592 | 1 | F | Addition to NR test case 7.3.2.1.3 FR2-FR2 RRC re-establishment without serving cell timing | 16.4.0 |
| 2020-06 | RAN#88 | R5-202836 | 0594 | 1 | F | Addition to NR test case 7.5.1.9 FR2 RLM scheduling restrictions | 16.4.0 |
| 2020-06 | RAN#88 | R5-202837 | 0578 | 1 | F | Correction to NR test case 8.3.1.1 handover to known cell | 16.4.0 |
| 2020-06 | RAN#88 | R5-202838 | 0597 | 1 | F | Addition of Default Configuration in Annex H | 16.4.0 |
| 2020-06 | RAN#88 | R5-202839 | 0628 | 1 | F | Update of Annex D.4.1.1 for parameters of BFD with 4RX | 16.4.0 |
| 2020-06 | RAN#88 | R5-202840 | 0630 | 1 | F | Update of Cell mapping in Annex E | 16.4.0 |
| 2020-06 | RAN#88 | R5-202841 | 0633 | 1 | F | Add auxiliary bands for RRM inter-frequency SA tests | 16.4.0 |
| 2020-06 | RAN#88 | R5-202909 | 0697 | 1 | F | Correction to Test Requirements in 4.4.1.1, Table 4.4.1.1.5-5 to include TT values from Annex F | 16.4.0 |
| 2020-06 | RAN#88 | R5-202910 | 0698 | 1 | F | Correction to Test Requirements in 6.4.1.1, Table 6.4.1.1.5-5 to include TT values from Annex F | 16.4.0 |
| 2020-06 | RAN#88 | R5-202911 | 0734 | 1 | F | Correction to Test Requirement for 6.4.3.1 by updating TT values from Annex F | 16.4.0 |
| 2020-06 | RAN#88 | R5-202912 | 0735 | 1 | F | Annex I RRM OTA procedures update to add RSRPB based UE Positioning Method for FR2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202913 | 0703 | 1 | F | Clarification of disabling Tx diversity for FR2 UE for FR2 RRM testing | 16.4.0 |
| 2020-06 | RAN#88 | R5-202914 | 0719 | 1 | F | Updates of FR2 MU and TT in TS 38.533 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202950 | 0693 | 1 | F | Correction to Message Exception for 4.6.2.5 EN-DC FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | 16.4.0 |
| 2020-06 | RAN#88 | R5-202951 | 0694 | 1 | F | Correction to Message Exception for 4.6.2.6 EN-DC FR1-FR1 event-triggered reporting in DRX with SSB time index detection | 16.4.0 |
| 2020-06 | RAN#88 | R5-202961 | 0690 | 1 | F | Correction to Test Applicability and Message Exception for 6.6.1.6 NR SA FR1 event-triggered reporting with gap in non-DRX with SSB index reading | 16.4.0 |
| 2020-06 | RAN#88 | R5-202969 | 0639 | 1 | F | Corrections to 4.7.2.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202970 | 0641 | 1 | F | Corrections to 4.7.3.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202971 | 0642 | 1 | F | Corrections to 4.7.3.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202972 | 0598 | 1 | F | Corrections to 6.7.1.2 and core spec alignment | 16.4.0 |
| 2020-06 | RAN#88 | R5-202973 | 0599 | 1 | F | Corrections to 6.7.2.1 and core spec alignment | 16.4.0 |
| 2020-06 | RAN#88 | R5-202974 | 0600 | 1 | F | Corrections to 6.7.2.2 and core spec alignment | 16.4.0 |
| 2020-06 | RAN#88 | R5-202975 | 0601 | 1 | F | Corrections to 6.7.3.1 and core spec alignment | 16.4.0 |
| 2020-06 | RAN#88 | R5-202976 | 0602 | 1 | F | Corrections to 6.7.3.2 and core spec alignment | 16.4.0 |
| 2020-06 | RAN#88 | R5-202977 | 0638 | 1 | F | Corrections to 4.7.1.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202978 | 0640 | 1 | F | Corrections to 4.7.2.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202982 | 0710 | 1 | F | Correction to FR1 UE transmit timing accuracy | 16.4.0 |
| 2020-06 | RAN#88 | R5-202983 | 0709 | 1 | F | Correction to EN-DC FR2 RRC-based DL active BWP switch in non-DRX in synchronous ENDC | 16.4.0 |
| 2020-06 | RAN#88 | R5-202984 | 0711 | 1 | F | Correction to FR2 UE transmit timing accuracy | 16.4.0 |
| 2020-06 | RAN#88 | R5-202985 | 0705 | 1 | F | Core alignment in 6.6.1.2 NR SA FR1 event-triggered reporting without gap in DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-202986 | 0713 | 1 | F | Correction to NR E-UTRA reselection tests | 16.4.0 |
| 2020-06 | RAN#88 | R5-202987 | 0715 | 1 | F | Correction to NR SA FR1 handover test cases | 16.4.0 |
| 2020-06 | RAN#88 | R5-202996 | 0604 | 1 | F | Update FR1 Test Tolerance of 4.5.2.5 E-UTRAN SCC in sync | 16.4.0 |
| 2020-06 | RAN#88 | R5-202997 | 0605 | 1 | F | Update FR1 Test Tolerance of 4.5.2.6 E-UTRAN SCC in async | 16.4.0 |
| 2020-06 | RAN#88 | R5-202998 | 0606 | 1 | F | Update FR1 Test Tolerance of 4.5.3.1 SCell activation 160ms | 16.4.0 |
| 2020-06 | RAN#88 | R5-202999 | 0607 | 1 | F | Update FR1 Test Tolerance of 4.5.3.2 SCell activation 320ms | 16.4.0 |
| 2020-06 | RAN#88 | R5-203000 | 0608 | 1 | F | Update FR1 Test Tolerance of 4.5.3.3 unknown SCell activation | 16.4.0 |
| 2020-06 | RAN#88 | R5-203091 | 0623 | 1 | F | Update of 4.5.3.0 minimum requirements for Scell activation | 16.4.0 |
| 2020-06 | RAN#88 | R5-203092 | 0613 | 1 | F | Update FR1 Test Tolerance of 6.5.4.1 UL carrier RRC reconfiguration delay | 16.4.0 |
| 2020-06 | RAN#88 | R5-203093 | 0629 | 1 | F | Update of FR1 Test Tolerance in Annex F | 16.4.0 |
| 2020-06 | RAN#88 | R5-203095 | 0609 | 1 | F | Addition FR1 Test Tolerance of 6.5.5.1 SSB BFD non-DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-203096 | 0610 | 1 | F | Addition FR1 Test Tolerance of 6.5.5.2 SSB BFD DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-203097 | 0611 | 1 | F | Update FR1 Test Tolerance of 6.5.5.3 CSI-RS BFD non-DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-203098 | 0612 | 1 | F | Update FR1 Test Tolerance of 6.5.5.4 CSI-RS BFD DRX | 16.4.0 |
| 2020-09 | RAN#89 | R5-203234 | 0747 | - | F | Update TC 8.5.2.1.1.1 with TT results and message contents | 16.5.0 |
| 2020-09 | RAN#89 | R5-203235 | 0748 | - | F | Update TC 8.5.2.2.1 with TT results and message contents | 16.5.0 |
| 2020-09 | RAN#89 | R5-203249 | 0758 | - | F | Annex E and F update with TT results for 8.5.2.x.x iRAT periodic reporting test cases | 16.5.0 |
| 2020-09 | RAN#89 | R5-203250 | 0759 | - | F | Annex E and F update with TT results for 4.7.4.x.x EN-DC L1-RSRP periodic reporting test cases | 16.5.0 |
| 2020-09 | RAN#89 | R5-203252 | 0761 | - | F | Void unnecessary TC 8.5.2.1.1.2 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203267 | 0765 | - | F | [EDITORIAL] Fix formatting issues throwing DOCX conversion errors in TS 38.533 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203268 | 0766 | - | F | Correction of test applicability for EN-DC event-triggered tests | 16.5.0 |
| 2020-09 | RAN#89 | R5-203269 | 0767 | - | F | Correction of test applicability for NR SA event-triggered tests | 16.5.0 |
| 2020-09 | RAN#89 | R5-203270 | 0768 | - | F | Adding annex table for iRAT measurement configuration | 16.5.0 |
| 2020-09 | RAN#89 | R5-203271 | 0769 | - | F | Clarification on the usage of auxiliary bands | 16.5.0 |
| 2020-09 | RAN#89 | R5-203272 | 0770 | - | F | Remove IE includeBeamMeasurements from 6.6.1.5 and 6.6.1.6 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203280 | 0772 | - | F | n26 addition to group of bands in 38.533 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203299 | 0773 | - | F | Corrected test description in non-DRX test case 6.5.1.2 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203300 | 0774 | - | F | Correction of comment for monitoringSymbolsWithinSlot | 16.5.0 |
| 2020-09 | RAN#89 | R5-203301 | 0775 | - | F | Correction of delta offset application for Cell 2 in test case 4.7.1.2.1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203303 | 0777 | - | F | PUSCH allocation correction in test case 6.4.3.1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203304 | 0778 | - | F | Correction to the RRC Reconfiguration message for measurements | 16.5.0 |
| 2020-09 | RAN#89 | R5-203306 | 0779 | - | F | Update extreme conditions limits in TC 4.7.1.2.1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203307 | 0780 | - | F | Update extreme conditions limits in TC 6.7.1.2.1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203308 | 0781 | - | F | Update extreme conditions limits in TC 4.7.1.2.1 and TC 6.7.1.2.1 - Annex F | 16.5.0 |
| 2020-09 | RAN#89 | R5-203309 | 0782 | - | F | Correction power offset for Band Group F | 16.5.0 |
| 2020-09 | RAN#89 | R5-203317 | 0786 | - | F | Removing SearchSpaceId and ControlResourceSetId modification | 16.5.0 |
| 2020-09 | RAN#89 | R5-203319 | 0788 | - | F | Unification of cell frequency for FR1-FR1 test cases | 16.5.0 |
| 2020-09 | RAN#89 | R5-203732 | 0790 | - | F | Set ss-sinr request to false for EN-DC RRM measurement accuracy tests | 16.5.0 |
| 2020-09 | RAN#89 | R5-203733 | 0791 | - | F | Set ss-sinr request to false for NR SA RRM measurement accuracy tests | 16.5.0 |
| 2020-09 | RAN#89 | R5-203832 | 0798 | - | F | Update of 4.5.6.1.0 minimum requirement for DCI-based BWP switch | 16.5.0 |
| 2020-09 | RAN#89 | R5-203835 | 0801 | - | F | Update of 4.5.6.2.0 minimum requirement for RRC-based BWP switch | 16.5.0 |
| 2020-09 | RAN#89 | R5-203837 | 0803 | - | F | Update of FR1 Test Tolerance in 6.3.2.1.3 RRC re-establishment without serving cell timing | 16.5.0 |
| 2020-09 | RAN#89 | R5-203838 | 0804 | - | F | Addition of 6.5.2.0 minimum requirement for interruptions during measurements on NR SCC | 16.5.0 |
| 2020-09 | RAN#89 | R5-203839 | 0805 | - | F | Update of 6.5.2.1 interruptions during measurements on deactivated NR SCC | 16.5.0 |
| 2020-09 | RAN#89 | R5-203840 | 0806 | - | F | Addition of new Test Case 6.5.3.1 SCell activation and deactivation for 160ms cycle | 16.5.0 |
| 2020-09 | RAN#89 | R5-203841 | 0807 | - | F | Addition of new Test Case 6.5.3.2 SCell activation and deactivation for 320ms cycle | 16.5.0 |
| 2020-09 | RAN#89 | R5-203842 | 0808 | - | F | Addition of new Test Case 6.5.3.3 SCell activation and deactivation of unknown SCell | 16.5.0 |
| 2020-09 | RAN#89 | R5-203847 | 0813 | - | F | Update of 6.5.6.1.0 minimum requirement for DCI-based BWP switch | 16.5.0 |
| 2020-09 | RAN#89 | R5-203850 | 0816 | - | F | Update of 6.5.6.2.0 minimum requirement for RRC-based BWP switch | 16.5.0 |
| 2020-09 | RAN#89 | R5-203853 | 0819 | - | F | Correction to FR1 intra-frequency measurement with gap test cases | 16.5.0 |
| 2020-09 | RAN#89 | R5-203855 | 0821 | - | F | Update of Cell mapping in Annex E | 16.5.0 |
| 2020-09 | RAN#89 | R5-204034 | 0827 | - | F | CR to 38.533 on RRM Test Cases in clause 7.5.3 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204036 | 0828 | - | F | CR to 38.533 on RRM Test Cases in clause 7.5.7 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204068 | 0829 | - | F | Correction to FR2 timing advance adjustment accuracy | 16.5.0 |
| 2020-09 | RAN#89 | R5-204069 | 0830 | - | F | Correction to EN-DC FR2 interruptions at transitions between active and non-active during DRX | 16.5.0 |
| 2020-09 | RAN#89 | R5-204070 | 0831 | - | F | Addition of E-UTRA Test frequency selection rule for IRAT test cases | 16.5.0 |
| 2020-09 | RAN#89 | R5-204071 | 0832 | - | F | Correction on event-triggered reporting with gap in DRX and DRX configuration settings for NR serving cell | 16.5.0 |
| 2020-09 | RAN#89 | R5-204072 | 0833 | - | F | Correction to EN-DC and SA FR1-FR1 SS-RSRP and SS-RSRQ periodical measurement accuracy | 16.5.0 |
| 2020-09 | RAN#89 | R5-204073 | 0834 | - | F | Correction to SS-RSRQ periodical measurement accuracy | 16.5.0 |
| 2020-09 | RAN#89 | R5-204075 | 0836 | - | F | Correction to message configuration for radio link monitoring tests | 16.5.0 |
| 2020-09 | RAN#89 | R5-204076 | 0837 | - | F | Correction to NR SA FR1 E-UTRA RRC connection release with redirection | 16.5.0 |
| 2020-09 | RAN#89 | R5-204077 | 0838 | - | F | Correction to SRS PeriodicityAndOffset for Timing Advance tests | 16.5.0 |
| 2020-09 | RAN#89 | R5-204079 | 0840 | - | F | Correction to CSI-RS for tracking for TDD FR2 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204116 | 0841 | - | F | Correction of EN-DC FR2 SSB-based L1-RSRP measurement test case 5.6.3.1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204118 | 0843 | - | F | Correction of EN-DC FR2 CSI-RS-based L1-RSRP measurement test case 5.6.3.3 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204119 | 0844 | - | F | Correction of EN-DC FR2 CSI-RS-based L1-RSRP measurement test case 5.6.3.4 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204122 | 0847 | - | F | Correction of NR SA FR2 SSB-based L1-RSRP measurement test case 7.6.3.1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204123 | 0848 | - | F | Correction of NR SA FR2 SSB-based L1-RSRP measurement test case 7.6.3.2 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204124 | 0849 | - | F | Correction of NR SA FR2 CSI-RS-based L1-RSRP measurement test case 7.6.3.3 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204125 | 0850 | - | F | Correction of NR SA FR2 CSI-RS-based L1-RSRP measurement test case 7.6.3.4 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204126 | 0851 | - | F | Correction of minimum conformance requirements 4.6.4.0 for EN-DC FR1 L1-RSRP measurement test cases | 16.5.0 |
| 2020-09 | RAN#89 | R5-204127 | 0852 | - | F | Correction of minimum conformance requirements 5.6.3.0 for EN-DC FR2 L1-RSRP measurement test cases | 16.5.0 |
| 2020-09 | RAN#89 | R5-204128 | 0853 | - | F | Correction of minimum conformance requirements 7.6.3.0 for SA FR2 L1-RSRP measurement test cases | 16.5.0 |
| 2020-09 | RAN#89 | R5-204220 | 0854 | - | F | Update missing SMTC and SSB configurations in H.2 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204221 | 0855 | - | F | Update missing SMTC and SSB configurations in H.3 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204222 | 0856 | - | F | Editorial: correct table placement in H.2 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204285 | 0859 | - | F | Correction to Message Exceptions for 4.6.2.5 EN-DC FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | 16.5.0 |
| 2020-09 | RAN#89 | R5-204337 | 0874 | - | F | Addition of 4.5.5.0 minimum requirement for BFR | 16.5.0 |
| 2020-09 | RAN#89 | R5-204730 | 0792 | 1 | F | Update of PDSCH RMC in Annex A.1.1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204733 | 0793 | 1 | F | Correction of RRM Test Case 6.1.1.1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204734 | 0824 | 1 | F | Correction of RRM Test Case 6.1.1.2 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204735 | 0825 | 1 | F | Correction of RRM Test Case 7.1.1.1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204736 | 0826 | 1 | F | Correction of RRM Test Case 7.1.1.2 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204775 | 0751 | 1 | F | Update TC 4.7.4.1.2 with TT results and message contents | 16.5.0 |
| 2020-09 | RAN#89 | R5-204776 | 0753 | 1 | F | Update TC 4.7.4.2.2 with TT results and message contents | 16.5.0 |
| 2020-09 | RAN#89 | R5-204777 | 0771 | 1 | F | Correction of measurement gap configuration for intra-frequency even-triggered tests needing gap | 16.5.0 |
| 2020-09 | RAN#89 | R5-204778 | 0755 | 1 | F | Update TC 6.7.4.1.2 with TT results and message contents | 16.5.0 |
| 2020-09 | RAN#89 | R5-204779 | 0757 | 1 | F | Update TC 6.7.4.2.2 with TT results and message contents | 16.5.0 |
| 2020-09 | RAN#89 | R5-204780 | 0762 | 1 | F | Correction to the test procedure of NR SA event triggered tests | 16.5.0 |
| 2020-09 | RAN#89 | R5-204781 | 0763 | 1 | F | Correction to the test procedure of NR SA event triggered tests inter-freq | 16.5.0 |
| 2020-09 | RAN#89 | R5-204782 | 0764 | 1 | F | Correction to the test procedure of NR SA event triggered tests inter-RAT | 16.5.0 |
| 2020-09 | RAN#89 | R5-204783 | 0749 | 1 | F | Update TC 8.5.2.3.1 with TT results and message contents | 16.5.0 |
| 2020-09 | RAN#89 | R5-204784 | 0760 | 1 | F | Annex E and F update with TT results for 6.7.4.x.x NR SA L1-RSRP periodic reporting test cases | 16.5.0 |
| 2020-09 | RAN#89 | R5-204785 | 0785 | 1 | F | Correction in A.1.4 CSI-RS for nzp-CSI-RS-ResourceId values | 16.5.0 |
| 2020-09 | RAN#89 | R5-204786 | 0820 | 1 | F | Update of FR1 Test Tolerance in Annex F | 16.5.0 |
| 2020-09 | RAN#89 | R5-204787 | 0823 | 1 | F | Correction to default configurations in Annex H | 16.5.0 |
| 2020-09 | RAN#89 | R5-204872 | 0857 | 1 | F | Correction to Test Procedure and sync to latest RAN4 in 4.4.1.1 EN-DC FR1 UE transmit timing accuracy | 16.5.0 |
| 2020-09 | RAN#89 | R5-204873 | 0858 | 1 | F | Correction to Test Procedure and sync to latest RAN4 in 6.4.1.1 NR SA FR1 UE transmit timing accuracy | 16.5.0 |
| 2020-09 | RAN#89 | R5-204874 | 0866 | 1 | F | Correction to 6.5.1.1 NR SA FR1 RLM OOS test with SSB-based RLM RS in non-DRX mode, sync to latest RAN4 Core Spec | 16.5.0 |
| 2020-09 | RAN#89 | R5-204875 | 0867 | 1 | F | Correction to 6.5.1.2 NR SA FR1 RLM IS test with SSB-based RLM RS in non-DRX mode, sync to latest RAN4 Core Spec | 16.5.0 |
| 2020-09 | RAN#89 | R5-204876 | 0868 | 1 | F | Correction to 6.5.1.3 NR SA FR1 RLM OOS test with SSB-based RLM RS in DRX mode, sync to latest RAN4 Core Spec | 16.5.0 |
| 2020-09 | RAN#89 | R5-204877 | 0869 | 1 | F | Correction to 6.5.1.4 NR SA FR1 RLM IS test with SSB-based RLM RS in DRX mode, sync to latest RAN4 Core Spec | 16.5.0 |
| 2020-09 | RAN#89 | R5-204878 | 0750 | 1 | F | Update TC 4.7.4.1.1 with TT results and message contents | 16.5.0 |
| 2020-09 | RAN#89 | R5-204879 | 0752 | 1 | F | Update TC 4.7.4.2.1 with TT results and message contents | 16.5.0 |
| 2020-09 | RAN#89 | R5-204880 | 0754 | 1 | F | Update TC 6.7.4.1.1 with TT results and message contents | 16.5.0 |
| 2020-09 | RAN#89 | R5-204881 | 0756 | 1 | F | Update TC 6.7.4.2.1 with TT results and message contents | 16.5.0 |
| 2020-09 | RAN#89 | R5-204882 | 0776 | 1 | F | Gap Pattern Id correction in section 6.6.2 to align to core spec 38.133 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204883 | 0787 | 1 | F | Test Procedure correction in test case 6.5.1.3 for definition of Point C | 16.5.0 |
| 2020-09 | RAN#89 | R5-204884 | 0789 | 1 | F | Addition of new RRM Test Cases in clause 7.5.5 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204885 | 0783 | 1 | F | CR to update the RRM FR2 MUs in Annex F | 16.5.0 |
| 2020-09 | RAN#89 | R5-204886 | 0784 | 1 | F | Add information on the AoA change during test for RRM FR2 tests | 16.5.0 |
| 2020-09 | RAN#89 | R5-204941 | 0835 | 1 | F | Correction to connection diagram for radio link monitoring tests | 16.5.0 |
| 2020-09 | RAN#89 | R5-204942 | 0842 | 1 | F | Correction of EN-DC FR2 SSB-based L1-RSRP measurement test case 5.6.3.2 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204943 | 0845 | 1 | F | Correction of FR2 condition tables in Annex B | 16.5.0 |
| 2020-09 | RAN#89 | R5-204944 | 0846 | 1 | F | Introduction of band n259 in clause 3 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204985 | 0794 | 1 | F | Update of FR1 Test Tolerance in 4.5.5.1 SSB based BFR in non-DRX | 16.5.0 |
| 2020-09 | RAN#89 | R5-204986 | 0795 | 1 | F | Update of FR1 Test Tolerance in 4.5.5.2 SSB based BFR in DRX | 16.5.0 |
| 2020-09 | RAN#89 | R5-204987 | 0796 | 1 | F | Addition of new Test Case 4.5.5.3 CSI-RS based BFR non-DRX with FR1 Test Tolerance | 16.5.0 |
| 2020-09 | RAN#89 | R5-204988 | 0797 | 1 | F | Update of FR1 Test Tolerance in 4.5.5.4 CSI-RS based BFR in DRX | 16.5.0 |
| 2020-09 | RAN#89 | R5-204989 | 0799 | 1 | F | Update of FR1 Test Tolerance in 4.5.6.1.1 DCI-based BWP switch | 16.5.0 |
| 2020-09 | RAN#89 | R5-204990 | 0800 | 1 | F | Update of FR1 Test Tolerance in 4.5.6.1.2 DCI-based BWP switch with SCell | 16.5.0 |
| 2020-09 | RAN#89 | R5-204991 | 0802 | 1 | F | Update of FR1 Test Tolerance in 4.5.6.2.1 RRC-based BWP switch | 16.5.0 |
| 2020-09 | RAN#89 | R5-204992 | 0809 | 1 | F | Update of FR1 Test Tolerance in 6.5.5.1 SSB based BFR in non-DRX | 16.5.0 |
| 2020-09 | RAN#89 | R5-204993 | 0810 | 1 | F | Update of FR1 Test Tolerance in 6.5.5.2 SSB based BFR in DRX | 16.5.0 |
| 2020-09 | RAN#89 | R5-204994 | 0811 | 1 | F | Update of FR1 Test Tolerance in 6.5.5.3 CSI-RS based BFR in non-DRX | 16.5.0 |
| 2020-09 | RAN#89 | R5-204995 | 0812 | 1 | F | Update of FR1 Test Tolerance in 6.5.5.4 CSI-RS based BFR in DRX | 16.5.0 |
| 2020-09 | RAN#89 | R5-204996 | 0814 | 1 | F | Update of FR1 Test Tolerance in 6.5.6.1.1 DCI-based BWP switch with SCell | 16.5.0 |
| 2020-09 | RAN#89 | R5-204997 | 0815 | 1 | F | Addition of new Test Case 6.5.6.1.2 DCI-based BWP switch with FR1 Test Tolerance | 16.5.0 |
| 2020-09 | RAN#89 | R5-204998 | 0817 | 1 | F | Update of FR1 Test Tolerance in 6.5.6.2.1 RRC-based BWP switch | 16.5.0 |
| 2020-09 | RAN#89 | R5-204999 | 0818 | 1 | F | Update of test parameters in 6.3.1.4 inter-RAT handover | 16.5.0 |
| 2020-09 | RAN#89 | R5-205000 | 0822 | 1 | F | Update of default configurations for RMC scheduling in Annex A | 16.5.0 |
| 2020-12 | RAN#90 | R5-205051 | 0879 | - | F | Message contents for RRM iRAT | 16.6.0 |
| 2020-12 | RAN#90 | R5-205059 | 0884 | - | F | Message contents for RRM PSCell addition | 16.6.0 |
| 2020-12 | RAN#90 | R5-205063 | 0885 | - | F | Update RRM 8.5.2.1.2 including TT analysis results | 16.6.0 |
| 2020-12 | RAN#90 | R5-205068 | 0890 | - | F | Correction of the measObjectID, measID and reportConfigID in Annex H | 16.6.0 |
| 2020-12 | RAN#90 | R5-205070 | 0892 | - | F | Remove includeBeamMeasurements requirement for TC4.6.1.5 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205257 | 0907 | - | F | NR Band n53 addition to RRM group of bands | 16.6.0 |
| 2020-12 | RAN#90 | R5-205296 | 0914 | - | F | Add FR2 Fading MU values | 16.6.0 |
| 2020-12 | RAN#90 | R5-205813 | 0920 | - | F | Correction of EN-DC FR2 Inter-frequency measurements test case 5.6.2.1 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205814 | 0921 | - | F | Correction of EN-DC FR2 Inter-frequency measurements test case 5.6.2.2 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205815 | 0922 | - | F | Correction of EN-DC FR2 Inter-frequency measurements test case 5.6.2.3 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205816 | 0923 | - | F | Correction of EN-DC FR2 Inter-frequency measurements test case 5.6.2.4 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205817 | 0924 | - | F | Correction of EN-DC FR2 Inter-frequency measurements test case 5.6.2.5 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205818 | 0925 | - | F | Correction of EN-DC FR2 Inter-frequency measurements test case 5.6.2.6 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205819 | 0926 | - | F | Correction of EN-DC FR2 Inter-frequency measurements test case 5.6.2.7 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205820 | 0927 | - | F | Correction of EN-DC FR2 Inter-frequency measurements test case 5.6.2.8 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205821 | 0928 | - | F | Correction of NR SA FR2 Inter-frequency measurements test case 7.6.2.1 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205822 | 0929 | - | F | Correction of NR SA FR2 Inter-frequency measurements test case 7.6.2.2 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205823 | 0930 | - | F | Correction of NR SA FR2 Inter-frequency measurements test case 7.6.2.3 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205824 | 0931 | - | F | Correction of NR SA FR2 Inter-frequency measurements test case 7.6.2.4 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205826 | 0933 | - | F | Correction of NR SA FR2 Inter-frequency measurements test case 7.6.2.6 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205827 | 0934 | - | F | Correction of NR SA FR2 Inter-frequency measurements test case 7.6.2.7 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205828 | 0935 | - | F | Correction of NR SA FR2 Inter-frequency measurements test case 7.6.2.8 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205834 | 0939 | - | F | Correction of message content exceptions in Annex H | 16.6.0 |
| 2020-12 | RAN#90 | R5-205953 | 0946 | - | F | Update of FR1 TT in 8.4.1.1 inter-RAT SFTD in non-DRX | 16.6.0 |
| 2020-12 | RAN#90 | R5-205954 | 0947 | - | F | Update of FR1 TT in 8.4.1.2 inter-RAT SFTD in DRX | 16.6.0 |
| 2020-12 | RAN#90 | R5-205960 | 0953 | - | F | Addition of test case 5.5.1.9 EN-DC FR2 radio link monitoring UE scheduling restrictions | 16.6.0 |
| 2020-12 | RAN#90 | R5-205961 | 0954 | - | F | Update of test case 5.5.3.1 EN-DC FR2 SCell activation and deactivation | 16.6.0 |
| 2020-12 | RAN#90 | R5-205963 | 0956 | - | F | Update of 7.3.2.1.3 SA FR2 RRC re-establishment without serving cell timing | 16.6.0 |
| 2020-12 | RAN#90 | R5-205967 | 0960 | - | F | Update of Annex E for cell mapping | 16.6.0 |
| 2020-12 | RAN#90 | R5-205975 | 0964 | - | F | Correction to default configuration in Annex H | 16.6.0 |
| 2020-12 | RAN#90 | R5-205976 | 0965 | - | F | Correction to SSB reference configuration | 16.6.0 |
| 2020-12 | RAN#90 | R5-206100 | 0968 | - | F | Correction to EN-DC FR1 interruptions at transitions between active and non-active during DRX | 16.6.0 |
| 2020-12 | RAN#90 | R5-206102 | 0970 | - | F | Correction to FR1 NSA SS-SINR measurement TCs | 16.6.0 |
| 2020-12 | RAN#90 | R5-206105 | 0973 | - | F | Correction to message exception for inter-RAT reselection TCs | 16.6.0 |
| 2020-12 | RAN#90 | R5-206106 | 0974 | - | F | Editorial correction of RSRP tests | 16.6.0 |
| 2020-12 | RAN#90 | R5-206111 | 0976 | - | F | Update of Annex C.1.3 of default connection setup | 16.6.0 |
| 2020-12 | RAN#90 | R5-206156 | 0977 | - | F | Addition of missing PRACH configuration 4 for FR1 and FR2 applicability | 16.6.0 |
| 2020-12 | RAN#90 | R5-206214 | 0979 | - | F | Alignment test parameter table with TS 38.133 in SA cell reselection test case | 16.6.0 |
| 2020-12 | RAN#90 | R5-206215 | 0980 | - | F | Alignment test parameter table with TS 38.133 in SA handover test case | 16.6.0 |
| 2020-12 | RAN#90 | R5-206216 | 0981 | - | F | Addition RLM IE into event trigger reporting CSI-RS based RLM | 16.6.0 |
| 2020-12 | RAN#90 | R5-206217 | 0982 | - | F | Addition RLM IE into event trigger reporting SSB based | 16.6.0 |
| 2020-12 | RAN#90 | R5-206219 | 0984 | - | F | Update message content section in SS-SINR test cases | 16.6.0 |
| 2020-12 | RAN#90 | R5-206221 | 0985 | - | F | Update to Section 5.4.1.1 NSA FR2 Transmit Timing Accuracy | 16.6.0 |
| 2020-12 | RAN#90 | R5-206222 | 0986 | - | F | Update to Section 5.4.3.1 NSA FR2 Timing Advance Accuracy | 16.6.0 |
| 2020-12 | RAN#90 | R5-206606 | 0998 | - | F | Update of CSI-RS RMC configuration in Annex A | 16.6.0 |
| 2020-12 | RAN#90 | R5-206672 | 0918 | 1 | F | Editorial CR to clarify SS-RSRP dBm / 15 kHz for PRACH tests | 16.6.0 |
| 2020-12 | RAN#90 | R5-206673 | 0941 | 1 | F | Correction of RRM TC 4.7.2.1 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206674 | 0943 | 1 | F | Update of message contents for EN-DC RLM test case | 16.6.0 |
| 2020-12 | RAN#90 | R5-206675 | 0969 | 1 | F | Correction to FR1 Inter frequency Event triggered Reporting test cases | 16.6.0 |
| 2020-12 | RAN#90 | R5-206677 | 0987 | 1 | F | Update to Message Exceptions for 4.3.2.2.2 EN-DC Non Contention Based Random Access | 16.6.0 |
| 2020-12 | RAN#90 | R5-206678 | 0989 | 1 | F | Update to 4.5.1.1 NSA FR1 OOS RLM to reword the Note for proper application of SNR for 4Rx Vs 2Rx | 16.6.0 |
| 2020-12 | RAN#90 | R5-206679 | 0990 | 1 | F | Update to 4.5.1.2 NSA FR1 IS RLM to reword the Note for proper application of SNR for 4Rx Vs 2Rx | 16.6.0 |
| 2020-12 | RAN#90 | R5-206680 | 0991 | 1 | F | Update to 4.5.1.3 NSA FR1 OOS RLM in DRX to reword the Note for proper application of SNR for 4Rx Vs 2Rx | 16.6.0 |
| 2020-12 | RAN#90 | R5-206681 | 0992 | 1 | F | Update to 4.5.1.4 NSA FR1 IS RLM in DRX to reword the Note for proper application of SNR for 4Rx Vs 2Rx | 16.6.0 |
| 2020-12 | RAN#90 | R5-206682 | 0993 | 1 | F | Update to 6.5.1.1 SA FR1 OOS RLM to reword the Note for proper application of SNR for 4Rx Vs 2Rx | 16.6.0 |
| 2020-12 | RAN#90 | R5-206683 | 0994 | 1 | F | Update to 6.5.1.2 SA FR1 IS RLM to reword the Note for proper application of SNR for 4Rx Vs 2Rx | 16.6.0 |
| 2020-12 | RAN#90 | R5-206684 | 0995 | 1 | F | Update to 6.5.1.3 SA FR1 OOS RLM in DRX to reword the Note for proper application of SNR for 4Rx Vs 2Rx | 16.6.0 |
| 2020-12 | RAN#90 | R5-206685 | 0996 | 1 | F | Update to 6.5.1.4 SA FR1 IS RLM in DRX to reword the Note for proper application of SNR for 4Rx Vs 2Rx | 16.6.0 |
| 2020-12 | RAN#90 | R5-206686 | 0952 | 1 | F | Update of 5.5.1 EN-DC FR2 radio link monitoring | 16.6.0 |
| 2020-12 | RAN#90 | R5-206687 | 0955 | 1 | F | Update of 5.5.5 EN-DC FR2 Beam failure detection | 16.6.0 |
| 2020-12 | RAN#90 | R5-206688 | 0891 | 1 | F | Corrections to re-selection test procedure | 16.6.0 |
| 2020-12 | RAN#90 | R5-206689 | 0896 | 1 | F | Correction of RRM TC 6.1.1.2 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206690 | 0897 | 1 | F | Correction of RRM TC 6.1.2.1 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206691 | 0899 | 1 | F | Correction of RRM TC 6.3.2.2.2 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206692 | 0910 | 1 | F | Correction of RRM TC 6.5.5.3 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206693 | 0911 | 1 | F | Correction of RRM TC 6.5.5.4 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206694 | 0919 | 1 | F | Editorial CR to clarify SS-RSRP dBm / 15 kHz for SA PRACH tests | 16.6.0 |
| 2020-12 | RAN#90 | R5-206695 | 0942 | 1 | F | Correction of RRM TC 6.7.2.1 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206696 | 0961 | 1 | F | Update of FR1 TT in 6.1.1.1 intra-freq re-selection | 16.6.0 |
| 2020-12 | RAN#90 | R5-206697 | 0962 | 1 | F | Correction of clause 6.1 cell re-selection | 16.6.0 |
| 2020-12 | RAN#90 | R5-206698 | 0967 | 1 | F | Correction to NR SA RLM out-of-sync test cases | 16.6.0 |
| 2020-12 | RAN#90 | R5-206699 | 0983 | 1 | F | Clarification in message exception for SA RLM out of synch test cases in non DRX | 16.6.0 |
| 2020-12 | RAN#90 | R5-206700 | 0900 | 1 | F | Correction of RRM TC 7.1.1.2 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206701 | 0932 | 1 | F | Correction of NR SA FR2 Inter-frequency measurements test case 7.6.2.5 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206702 | 0936 | 1 | F | Correction of NR SA FR2 RRC re-establishment test case 7.3.2.1.1 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206703 | 0937 | 1 | F | Correction of NR SA FR2-FR2 RRC re-establishment test case 7.3.2.1.2 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206704 | 0957 | 1 | F | Update of clause 7.5.5 SA FR2 Beam failure detection | 16.6.0 |
| 2020-12 | RAN#90 | R5-206705 | 0958 | 1 | F | Update of clause 7.6.1 SA FR2 Intra-frequency measurements | 16.6.0 |
| 2020-12 | RAN#90 | R5-206706 | 0889 | 1 | F | Correction of the message contents for RRM re-selection test cases | 16.6.0 |
| 2020-12 | RAN#90 | R5-206707 | 0917 | 1 | F | Correction to the aggregation level in the CORESET for RMC scheduling | 16.6.0 |
| 2020-12 | RAN#90 | R5-206708 | 0938 | 1 | F | Correction of clause 3.3 Abbreviations | 16.6.0 |
| 2020-12 | RAN#90 | R5-206709 | 0963 | 1 | F | Update of Annex H for cell re-selection | 16.6.0 |
| 2020-12 | RAN#90 | R5-206797 | 0882 | 1 | F | Complete RRM 4.5.7.1 including TT analysis results | 16.6.0 |
| 2020-12 | RAN#90 | R5-206798 | 0875 | 1 | F | Complete RRM 6.7.5.1 including TT analysis results | 16.6.0 |
| 2020-12 | RAN#90 | R5-206799 | 0916 | 1 | F | Add FR2 PRACH Test cases 7.3.2.2.1 and 7.3.2.2.2 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206800 | 0944 | 1 | F | Update of FR1 TT in 8.2.1.1 inter-RAT cell re-selection | 16.6.0 |
| 2020-12 | RAN#90 | R5-206801 | 0945 | 1 | F | Update of FR1 TT in 8.3.1.1 inter-RAT handover | 16.6.0 |
| 2020-12 | RAN#90 | R5-206802 | 0948 | 1 | F | Update of FR1 TT in 8.4.2.1 inter-RAT event-triggered reporting without SSB time index in non-DRX | 16.6.0 |
| 2020-12 | RAN#90 | R5-206803 | 0949 | 1 | F | Update of FR1 TT in 8.4.2.2 inter-RAT event-triggered reporting without SSB time index in DRX | 16.6.0 |
| 2020-12 | RAN#90 | R5-206804 | 0950 | 1 | F | Update of FR1 TT in 8.4.2.3 inter-RAT event-triggered reporting with SSB time index in non-DRX | 16.6.0 |
| 2020-12 | RAN#90 | R5-206805 | 0951 | 1 | F | Update of FR1 TT in 8.4.2.4 inter-RAT event-triggered reporting with SSB time index in DRX | 16.6.0 |
| 2020-12 | RAN#90 | R5-206806 | 0878 | 1 | F | Annex E and F iRAT measurement accuracy tests | 16.6.0 |
| 2020-12 | RAN#90 | R5-206807 | 0888 | 1 | F | Annex E and F iRAT FR2 measurement accuracy tests | 16.6.0 |
| 2020-12 | RAN#90 | R5-206808 | 0959 | 1 | F | Update of Annex F for Test Tolerance | 16.6.0 |
| 2020-12 | RAN#90 | R5-206834 | 0915 | 1 | F | Update FR2 Downlink and Es/Noc MU values | 16.6.0 |
| 2020-12 | RAN#90 | R5-206843 | 0876 | 1 | F | Complete RRM 6.7.6.1 including TT analysis results | 16.6.0 |
| 2020-12 | RAN#90 | R5-206844 | 0886 | 1 | F | Update RRM 8.5.2.2.2 including TT analysis results | 16.6.0 |
| 2020-12 | RAN#90 | R5-206871 | 0894 | 1 | F | Correction of RRM TC 4.3.2.2.2 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206872 | 0905 | 1 | F | New DRX configuration for RLM TC | 16.6.0 |
| 2020-12 | RAN#90 | R5-206895 | 0971 | 1 | F | Correction to message configuration for NSA radio link monitoring CSI-RS Based in DRX mode tests | 16.6.0 |
| 2020-12 | RAN#90 | R5-206906 | 0880 | 1 | F | Complete RRM 4.7.5.1 including TT analysis results | 16.6.0 |
| 2020-12 | RAN#90 | R5-206907 | 0877 | 1 | F | Complete RRM 6.7.7.1 including TT analysis results | 16.6.0 |
| 2020-12 | RAN#90 | R5-206908 | 0972 | 1 | F | Correction to message configuration for SA radio link monitoring CSI RS Based RLM RS | 16.6.0 |
| 2020-12 | RAN#90 | R5-206909 | 0887 | 1 | F | Update RRM 8.5.2.3.2 including TT analysis results | 16.6.0 |
| 2020-12 | RAN#90 | R5-206910 | 0881 | 1 | F | Annex E and F 4.7.5.1 SFTD measurement accuracy test | 16.6.0 |
| 2021-03 | RAN#91 | R5-210134 | 0999 | - | F | Update of first preamble power for EN-DC TC 4.3.2.2.1 and 4.3.2.2.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210135 | 1000 | - | F | Update of first preamble power for SA TC 6.3.2.2.1 and 6.3.2.2.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210136 | 1001 | - | F | Update of UE initial state for SA RLM TC 6.5.1.x | 16.7.0 |
| 2021-03 | RAN#91 | R5-210137 | 1002 | - | F | Update of process delay for SFTD measurement TC 8.4.1.x | 16.7.0 |
| 2021-03 | RAN#91 | R5-210138 | 1003 | - | F | Update of cell frequency for TC 6.7.2.2.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210171 | 1004 | - | F | Editorial update of SS-RSRP for TC 6.7.1.2.1 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210173 | 1006 | - | F | Update of RRC message for TC 6.4.3.1 and 6.5.2.1 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210176 | 1009 | - | F | Update of Scell activation and CSI reporting time for EN-DC TC 5.5.3.1 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210177 | 1010 | - | F | Update of CSI reporting time for SA TC 7.5.3.1 and 7.5.3.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210178 | 1011 | - | F | Update of PRACH configuration for SA TC 7.3.2.2.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210179 | 1012 | - | F | Update of PRACH configuration for EN-DC TC 4.5.5.3 and 4.5.5.4 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210180 | 1013 | - | F | Update of PRACH configuration for EN-DC TC 5.5.5.x | 16.7.0 |
| 2021-03 | RAN#91 | R5-210181 | 1014 | - | F | Update of PRACH configuration for SA TC 6.5.5.3 and 6.5.5.4 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210182 | 1015 | - | F | Update of PRACH configuration for SA TC 7.5.5.x | 16.7.0 |
| 2021-03 | RAN#91 | R5-210183 | 1016 | - | F | Update of prach-ConfigurationIndex for FR1 PRACH configuration 4 in A.7 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210279 | 1017 | - | F | Addition of minimum conformance requirements for R16 NR HST cell re-selection in 6.1.1.0 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210281 | 1019 | - | F | Addition of minimum conformance requirements for R16 NR HST measurement in 6.6.1.0 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210283 | 1021 | - | F | Update of Annex H.2.1-3 for R16 NR HST | 16.7.0 |
| 2021-03 | RAN#91 | R5-210434 | 1025 | - | F | Correct cell mapping for 5.4.3.1 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210437 | 1027 | - | F | Annex E and F 4.5.7.1 PSCell addition test | 16.7.0 |
| 2021-03 | RAN#91 | R5-210440 | 1029 | - | F | Update of TT analysis results in Annex F for FR2 iRAT test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-210442 | 1030 | - | F | Update TT results for SS-RSRP measurement accuracy test cases chapter 4 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210443 | 1031 | - | F | Update TT results for SS-RSRP measurement accuracy test cases chapter 6 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210444 | 1032 | - | F | Update TT results for SS-RSRP measurement accuracy test cases Annex F | 16.7.0 |
| 2021-03 | RAN#91 | R5-210446 | 1033 | - | F | Update TT results for SS-RSRQ measurement accuracy test cases chapter 4 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210447 | 1034 | - | F | Update TT results for SS-RSRQ measurement accuracy test cases chapter 6 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210448 | 1035 | - | F | Update TT results for SS-RSRQ measurement accuracy test cases Annex F | 16.7.0 |
| 2021-03 | RAN#91 | R5-210456 | 1039 | - | F | Update Annex I to TS 38.533 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210457 | 1040 | - | F | Update NR frequency band groups for FR1 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210458 | 1041 | - | F | Correction RLM config for event triggered test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-210465 | 1046 | - | F | Editorial correction of title of clause 6 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210466 | 1047 | - | F | Corrections to 6.3.2.2.1 and 6.3.2.2.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210467 | 1048 | - | F | Corrections to 4.3.2.2.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210470 | 1049 | - | F | Clarification on SSB Index to use in the PUxCH-PowerControl for RRM tests with more than one SSB | 16.7.0 |
| 2021-03 | RAN#91 | R5-210471 | 1050 | - | F | Clarification on SSB Index to use in the PUxCH-PowerControl for RRM tests with more than one SSB - EN-DC | 16.7.0 |
| 2021-03 | RAN#91 | R5-210472 | 1051 | - | F | Editorial: correct title of PRACH test cases to match RRM work plan - EN-DC FR1 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210473 | 1052 | - | F | Editorial: correct title of PRACH test cases to match RRM work plan - SA FR1 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210474 | 1053 | - | F | Editorial: correct title of PRACH test cases to match RRM work plan - SA FR2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210475 | 1054 | - | F | Editorial: correct title of PRACH test cases to match RRM work plan - Annexes | 16.7.0 |
| 2021-03 | RAN#91 | R5-210507 | 1056 | - | F | Correction of test applicability for long DRX cycle related test cases in section 4 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210508 | 1057 | - | F | Correction of test applicability for long DRX cycle related test cases in section 5 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210509 | 1058 | - | F | Correction of test applicability for long DRX cycle related test cases in section 6 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210510 | 1059 | - | F | Correction of test applicability for long DRX cycle related test cases in section 7 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210511 | 1060 | - | F | Correction of test applicability for long DRX cycle related test cases in section 8 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210528 | 1062 | - | F | Correction to test procedure in test case 6.5.1.4 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210529 | 1063 | - | F | Correction to Interruptions during measurements on deactivated NR SCC test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-210530 | 1064 | - | F | Correction to message configuration for NSA CSI-RS-based RLM RS test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-210531 | 1065 | - | F | Correction to NR SA RLM out-of-sync test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-210532 | 1066 | - | F | Correction to NR SA FR1 RRC Re-establishment test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-210533 | 1067 | - | F | Correction to 4.5.1.2 and 6.5.1.2 PDCCH Aggregation Level | 16.7.0 |
| 2021-03 | RAN#91 | R5-210535 | 1069 | - | F | Correction to FR1 NSA SS-SINR measurement test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-210536 | 1070 | - | F | Correction to L1-RSRP test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-210537 | 1071 | - | F | Correction to the procedure to add a step for establishing SRB2 and DRB | 16.7.0 |
| 2021-03 | RAN#91 | R5-210538 | 1072 | - | F | Update of DRX configuration in FR1 Event-triggered Test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-210601 | 1073 | - | F | Clarification of BWP1 and BWP2 in 6.6.1.3, 6.6.1.4, 6.6.1.6 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210608 | 1077 | - | F | Correction in 4.5.2.1 and 4.5.2.2 test procedure | 16.7.0 |
| 2021-03 | RAN#91 | R5-210613 | 1079 | - | F | Correction in SIB5 for iRAT cell reselection | 16.7.0 |
| 2021-03 | RAN#91 | R5-210614 | 1080 | - | F | Correction in 6.1.2.2 test procedure | 16.7.0 |
| 2021-03 | RAN#91 | R5-210814 | 1085 | - | F | Finalise FR2 Timing MU values | 16.7.0 |
| 2021-03 | RAN#91 | R5-210816 | 1086 | - | F | Update of FR2 Tx Timing Test case 5.4.1.1 Test Tolerances | 16.7.0 |
| 2021-03 | RAN#91 | R5-210851 | 1089 | - | F | Update of FR1 TT for 4.5.5.1 and 4.5.5.2 SSB based LR | 16.7.0 |
| 2021-03 | RAN#91 | R5-210852 | 1090 | - | F | Update of FR1 TT for 4.5.5.3 and 4.5.5.4 CSI-RS based LR | 16.7.0 |
| 2021-03 | RAN#91 | R5-210853 | 1091 | - | F | Correction to 4.6.4.3 and 4.6.4.4 L1-RSRP reporting delay | 16.7.0 |
| 2021-03 | RAN#91 | R5-210857 | 1095 | - | F | Update of FR1 TT for 6.5.5.1 and 6.5.5.2 SSB based LR | 16.7.0 |
| 2021-03 | RAN#91 | R5-210858 | 1096 | - | F | Update of FR1 TT for 6.5.5.3 and 6.5.5.4 CSI-RS based LR | 16.7.0 |
| 2021-03 | RAN#91 | R5-210860 | 1098 | - | F | Correction to 6.6.4.3 and 6.6.4.4 L1-RSRP reporting delay | 16.7.0 |
| 2021-03 | RAN#91 | R5-210863 | 1101 | - | F | Update of 6.5.3.1 SCell activation and deactivation | 16.7.0 |
| 2021-03 | RAN#91 | R5-210864 | 1102 | - | F | Update of D.4 antenna configuration | 16.7.0 |
| 2021-03 | RAN#91 | R5-210866 | 1104 | - | F | Correction to CSI-RS RMC | 16.7.0 |
| 2021-03 | RAN#91 | R5-210867 | 1105 | - | F | Correction to default configuration on L1-RSRP reporting in Annex H | 16.7.0 |
| 2021-03 | RAN#91 | R5-210959 | 1113 | - | F | Update to 6.5.4.1 NR SA FR1 UE UL carrier RRC reconfiguration delay | 16.7.0 |
| 2021-03 | RAN#91 | R5-211049 | 1116 | - | F | Additional of default downlink level for FR2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211219 | 1124 | - | F | Correction of the minimum conformance requirements for Inter-RAT measurements 6.6.3.0 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211220 | 1125 | - | F | Addition of new RRM iRAT measurement Inter-RAT measurements test case 6.6.3.3 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211255 | 1131 | - | F | Clarification of BWP1 and BWP2 in 4.6.1.3, 4.6.1.4, 4.6.1.6 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211614 | 1007 | 1 | F | Update of Scell activation and CSI reporting time for EN-DC TC 4.5.3.1 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211615 | 1026 | 1 | F | Complete RRM 4.5.7.1 including TT analysis results | 16.7.0 |
| 2021-03 | RAN#91 | R5-211616 | 1087 | 1 | F | Update Test Tolerance for FR1 RLM Test Cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-211617 | 1092 | 1 | F | Correction to EN-DC radio link monitoring | 16.7.0 |
| 2021-03 | RAN#91 | R5-211618 | 1093 | 1 | F | Update of 4.5.3.1 SCell activation and deactivation | 16.7.0 |
| 2021-03 | RAN#91 | R5-211619 | 1108 | 1 | F | Update to 4.5.4.1 EN-DC FR1 UE UL carrier RRC reconfiguration delay | 16.7.0 |
| 2021-03 | RAN#91 | R5-211620 | 1109 | 1 | F | Update to 4.7.5.1 EN-DC FR1 SFTD measurement accuracy | 16.7.0 |
| 2021-03 | RAN#91 | R5-211621 | 1117 | 1 | F | Correction of applied TT for EN-DC FR1 L1-RSRP measurement test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-211622 | 1023 | 1 | F | Update 5.4.3.1 with TT analysis results | 16.7.0 |
| 2021-03 | RAN#91 | R5-211623 | 1119 | 1 | F | Correction of EN-DC FR2 inter-freq measurement test case 5.6.2.1 including TT | 16.7.0 |
| 2021-03 | RAN#91 | R5-211624 | 1120 | 1 | F | Correction of EN-DC FR2 inter-freq measurement test case 5.6.2.3 including TT | 16.7.0 |
| 2021-03 | RAN#91 | R5-211625 | 1008 | 1 | F | Update of CSI reporting time for SA TC 6.5.3.1 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211626 | 1094 | 1 | F | Correction to SA radio link monitoring | 16.7.0 |
| 2021-03 | RAN#91 | R5-211627 | 1110 | 1 | F | Update to 6.3.2.2.1 Contention based random access test in FR1 for NR standalone | 16.7.0 |
| 2021-03 | RAN#91 | R5-211628 | 1111 | 1 | F | Update to 6.3.2.2.2 Non-Contention based random access test in FR1 for NR standalone | 16.7.0 |
| 2021-03 | RAN#91 | R5-211629 | 1112 | 1 | F | Update to 6.5.2.1 NR SA FR1 interruptions during measurements on deactivated NR SCC | 16.7.0 |
| 2021-03 | RAN#91 | R5-211630 | 1114 | 1 | F | Update to 6.6.3.1 NR SA FR1 - E-UTRAN event-triggered reporting in non-DRX | 16.7.0 |
| 2021-03 | RAN#91 | R5-211631 | 1115 | 1 | F | Update to 6.6.3.2 NR SA FR1 - E-UTRAN event-triggered reporting in DRX | 16.7.0 |
| 2021-03 | RAN#91 | R5-211632 | 1118 | 1 | F | Correction of applied TT for SA FR1 L1-RSRP measurement test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-211633 | 1121 | 1 | F | Correction of SA FR2 inter-freq measurement test case 7.6.2.1 including TT | 16.7.0 |
| 2021-03 | RAN#91 | R5-211634 | 1122 | 1 | F | Correction of SA FR2 inter-freq measurement test case 7.6.2.3 including TT | 16.7.0 |
| 2021-03 | RAN#91 | R5-211635 | 1028 | 1 | F | Complete FR2 iRAT measurement accuracy test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-211636 | 1024 | 1 | F | Update Annex F for 5.4.3.1 and 7.4.3.1 with TT analysis results | 16.7.0 |
| 2021-03 | RAN#91 | R5-211637 | 1038 | 1 | F | Update DL AWGN MU for RRM FR2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211638 | 1084 | 1 | F | Update FR2 Downlink and Uplink MU values | 16.7.0 |
| 2021-03 | RAN#91 | R5-211639 | 1103 | 1 | F | Update of Annex F for Test Tolerance | 16.7.0 |
| 2021-03 | RAN#91 | R5-211640 | 1123 | 1 | F | Correction of MTSU and applied TT in Annex F | 16.7.0 |
| 2021-03 | RAN#91 | R5-211641 | 1130 | 1 | F | Addition of new RMCs and OCNGs into Annex A | 16.7.0 |
| 2021-03 | RAN#91 | R5-211722 | 1042 | 1 | F | Correction EN-DC FR1 timing tests | 16.7.0 |
| 2021-03 | RAN#91 | R5-211723 | 1043 | 1 | F | Corrections to 5.4.1.1 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211724 | 1061 | 1 | F | Clarification of SNR for 4RX UE in RLM Test Cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-211725 | 1076 | 1 | F | Correction in 4.7.3.2.1 test parameters | 16.7.0 |
| 2021-03 | RAN#91 | R5-211726 | 1005 | 1 | F | Update of RRC message for TC 6.3.1.4, 6.3.1.5 and 6.3.2.3.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211727 | 1045 | 1 | F | Corrections NR SA FR1 timing test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-211728 | 1097 | 1 | F | Correction to NR TC 6.6.3.2-Inter-RAT DRX | 16.7.0 |
| 2021-03 | RAN#91 | R5-211729 | 1099 | 1 | F | Update of 6.5.1.2 and 6.5.1.4 SSB based RLM | 16.7.0 |
| 2021-03 | RAN#91 | R5-211730 | 1083 | 1 | F | Clarification of BWP1 and BWP2 in 7.6.1.3 and 7.6.1.4 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211731 | 1044 | 1 | F | Addition of Serving Cell Config for RRM timing test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-211827 | 1018 | 1 | F | Addition of new test case 6.1.1.7 for R16 NR HST | 16.7.0 |
| 2021-03 | RAN#91 | R5-211828 | 1020 | 1 | F | Addition of new test case 6.6.1.7 for R16 NR HST | 16.7.0 |
| 2021-03 | RAN#91 | R5-211829 | 1022 | 1 | F | Addition of cell configuration mapping in Annex E for R16 NR HST | 16.7.0 |
| 2021-03 | RAN#91 | R5-211830 | 1126 | 1 | F | Correction of the minimum conformance requirements for E-UTRA - NR FR1 Cell reselection 8.2.1.0 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211831 | 1127 | 1 | F | Addition of new RRM E-UTRA - NR FR1 Cell reselection test case 8.2.1.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211832 | 1128 | 1 | F | Correction of the minimum conformance requirements for E-UTRA - NR Inter-RAT event triggered reporting tests 8.4.2.0 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211833 | 1129 | 1 | F | Addition of new RRM E-UTRA - NR Inter-RAT event triggered reporting test case 8.4.2.9 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211889 | 1036 | 1 | F | Update TT results for 6.3.1.1 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211890 | 1037 | 1 | F | Update TT results for 6.3.1.1 Annex F | 16.7.0 |
| 2021-06 | RAN#92 | R5-212024 | 1132 | - | F | Complete 5.7.1.1 with TT analysis results | 16.8.0 |
| 2021-06 | RAN#92 | R5-212025 | 1133 | - | F | Complete 5.7.2.1 with TT analysis results | 16.8.0 |
| 2021-06 | RAN#92 | R5-212026 | 1134 | - | F | Complete 5.7.3.1 with TT analysis results | 16.8.0 |
| 2021-06 | RAN#92 | R5-212256 | 1138 | - | F | Core alignment of TCI state of CSI-RS for TRS | 16.8.0 |
| 2021-06 | RAN#92 | R5-212257 | 1139 | - | F | Introducing additional band for n38 | 16.8.0 |
| 2021-06 | RAN#92 | R5-212265 | 1147 | - | F | Updates to DRX configuration in FR1 Event-triggered Test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-212266 | 1148 | - | F | Core alignment of BWP configuration parameter in 6.5.2.1 | 16.8.0 |
| 2021-06 | RAN#92 | R5-212267 | 1149 | - | F | Correction to CSI-RS based L1-RSRP measurement tests | 16.8.0 |
| 2021-06 | RAN#92 | R5-212612 | 1153 | - | F | Addition of new test case 6.6.4.5 for R16 NR HST | 16.8.0 |
| 2021-06 | RAN#92 | R5-212613 | 1154 | - | F | Addition of cell configuration mapping in Annex E for TC 6.6.4.5 | 16.8.0 |
| 2021-06 | RAN#92 | R5-212712 | 1158 | - | F | Remove obsolete message contents tables after Chapter 7 alignment | 16.8.0 |
| 2021-06 | RAN#92 | R5-212715 | 1161 | - | F | Correcting measurement accuracy iRAT test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-212716 | 1162 | - | F | Corrections to 5.4.1.1 | 16.8.0 |
| 2021-06 | RAN#92 | R5-212717 | 1163 | - | F | Corrections to 5.4.3.1 | 16.8.0 |
| 2021-06 | RAN#92 | R5-212718 | 1164 | - | F | Corrections to 4.4.1.1 | 16.8.0 |
| 2021-06 | RAN#92 | R5-212719 | 1165 | - | F | Corrections to 4.4.3.1 | 16.8.0 |
| 2021-06 | RAN#92 | R5-212720 | 1166 | - | F | Corrections to 6.4.1.1 | 16.8.0 |
| 2021-06 | RAN#92 | R5-212721 | 1167 | - | F | Corrections to 6.4.3.1 | 16.8.0 |
| 2021-06 | RAN#92 | R5-212726 | 1171 | - | F | Correct applicability of Interruption EN-DC test cases with CA | 16.8.0 |
| 2021-06 | RAN#92 | R5-212727 | 1172 | - | F | Correct applicability of Scell activation EN-DC test cases with CA | 16.8.0 |
| 2021-06 | RAN#92 | R5-212728 | 1173 | - | F | Correct applicability of Interruption SA test cases with CA | 16.8.0 |
| 2021-06 | RAN#92 | R5-212729 | 1174 | - | F | Correct applicability of Scell activation SA test cases with CA | 16.8.0 |
| 2021-06 | RAN#92 | R5-212731 | 1175 | - | F | Addition of RRM FR2 EN-DC PRACH test case 5.3.2.2.1 | 16.8.0 |
| 2021-06 | RAN#92 | R5-212732 | 1176 | - | F | Addition of RRM FR2 EN-DC PRACH test case 5.3.2.2.2 | 16.8.0 |
| 2021-06 | RAN#92 | R5-212737 | 1177 | - | F | Update to test applicability of RRM TC6.5.4.1 | 16.8.0 |
| 2021-06 | RAN#92 | R5-212745 | 1178 | - | F | Update to RRM Antenna connection for 4Rx capable Ues | 16.8.0 |
| 2021-06 | RAN#92 | R5-212895 | 1180 | - | F | Correction to FR1 EN-DC interruption during SCell measurement test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-212896 | 1181 | - | F | Correction to FR1 EN-DC beam failure recovery test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-212900 | 1185 | - | F | Correction to FR1 NR SA TC 6.5.3.1-SCell activation | 16.8.0 |
| 2021-06 | RAN#92 | R5-212901 | 1186 | - | F | Correction to FR1 NR SA beam failure recovery test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-212904 | 1189 | - | F | Correction to RMSI CORESET reference configuration in Annex A | 16.8.0 |
| 2021-06 | RAN#92 | R5-212905 | 1190 | - | F | Correction to CSI-RS reference configuration in annex A | 16.8.0 |
| 2021-06 | RAN#92 | R5-212906 | 1191 | - | F | Correction to TRS reference configuration in annex A | 16.8.0 |
| 2021-06 | RAN#92 | R5-212909 | 1194 | - | F | Correction to BWP reference configuration in Annex A | 16.8.0 |
| 2021-06 | RAN#92 | R5-212910 | 1195 | - | F | Correction of minimum requirements for L1-RSRP reporting test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-212911 | 1196 | - | F | Addition of FR1 EN-DC TC 4.6.4.5-HST L1-RSRP | 16.8.0 |
| 2021-06 | RAN#92 | R5-213183 | 1205 | - | F | Extension of Annex I for EIS spherical coverage points | 16.8.0 |
| 2021-06 | RAN#92 | R5-213227 | 1212 | - | F | Correction of SA FR2 inter-freq measurement test case 7.6.2.5 including TT | 16.8.0 |
| 2021-06 | RAN#92 | R5-213228 | 1213 | - | F | Correction of SA FR2 inter-freq measurement test case 7.6.2.6 including TT | 16.8.0 |
| 2021-06 | RAN#92 | R5-213229 | 1214 | - | F | Correction of SA FR2 inter-freq measurement test case 7.6.2.7 including TT | 16.8.0 |
| 2021-06 | RAN#92 | R5-213230 | 1215 | - | F | Correction of SA FR2 inter-freq measurement test case 7.6.2.8 including TT | 16.8.0 |
| 2021-06 | RAN#92 | R5-213240 | 1219 | - | F | Test frequency selection for Inter-frequency RRM test cases in Annex E | 16.8.0 |
| 2021-06 | RAN#92 | R5-213345 | 1222 | - | F | Update on iRAT measurement 6.6.3.2 test case | 16.8.0 |
| 2021-06 | RAN#92 | R5-213346 | 1223 | - | F | Update on Annex H.3.1 for iRAT measurement | 16.8.0 |
| 2021-06 | RAN#92 | R5-213351 | 1224 | - | F | Correction to Coreset and Search Space RMC for FR2 RRM | 16.8.0 |
| 2021-06 | RAN#92 | R5-213352 | 1225 | - | F | Correction to Message Exceptions for 6.3.2.2.2 Non-contention based random access test in FR1 | 16.8.0 |
| 2021-06 | RAN#92 | R5-213353 | 1226 | - | F | Correction to PDSCH RMC when DRX is configured | 16.8.0 |
| 2021-06 | RAN#92 | R5-213354 | 1227 | - | F | Correction to intra and inter frequency reselection test cases with asynchronous cells | 16.8.0 |
| 2021-06 | RAN#92 | R5-213847 | 1179 | 1 | F | Addition of inter-frequency measurements configuration for FR1 EN-DC BFD and RLM test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-213848 | 1183 | 1 | F | Addition of inter-frequency measurements configuration for FR1 NR SA BFD and RLM test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-213849 | 1184 | 1 | F | Correction to FR1 NR SA interruption during SCell measurement test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-213850 | 1193 | 1 | F | Correction to PRACH reference configuration in Annex H | 16.8.0 |
| 2021-06 | RAN#92 | R5-213921 | 1145 | 1 | F | Correction to test procedure for SSB-based L1-RSRP measurement reporting | 16.8.0 |
| 2021-06 | RAN#92 | R5-213922 | 1159 | 1 | F | Remove obsolete message contents tables after Chapter 7 alignment - EN-DC | 16.8.0 |
| 2021-06 | RAN#92 | R5-213923 | 1168 | 1 | F | Corrections to RLM test cases EN-DC | 16.8.0 |
| 2021-06 | RAN#92 | R5-213924 | 1141 | 1 | F | Correction to FR2 RRM test channel reference for Event triggered reporting test | 16.8.0 |
| 2021-06 | RAN#92 | R5-213925 | 1142 | 1 | F | Correction to FR2 RRM test channel reference for RLM and BFD tests | 16.8.0 |
| 2021-06 | RAN#92 | R5-213926 | 1143 | 1 | F | Correction to FR2 RRM test channel reference for UL timing test | 16.8.0 |
| 2021-06 | RAN#92 | R5-213927 | 1208 | 1 | F | Correction of EN-DC FR2 inter-freq measurement test case 5.6.2.5 including TT | 16.8.0 |
| 2021-06 | RAN#92 | R5-213928 | 1209 | 1 | F | Correction of EN-DC FR2 inter-freq measurement test case 5.6.2.6 including TT | 16.8.0 |
| 2021-06 | RAN#92 | R5-213929 | 1210 | 1 | F | Correction of EN-DC FR2 inter-freq measurement test case 5.6.2.7 including TT | 16.8.0 |
| 2021-06 | RAN#92 | R5-213930 | 1211 | 1 | F | Correction of EN-DC FR2 inter-freq measurement test case 5.6.2.8 including TT | 16.8.0 |
| 2021-06 | RAN#92 | R5-213931 | 1156 | 1 | F | Correct reference to cell mapping for SA test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-213932 | 1160 | 1 | F | Correcting typos in PRACH test case | 16.8.0 |
| 2021-06 | RAN#92 | R5-213933 | 1169 | 1 | F | Corrections to RLM test cases SA | 16.8.0 |
| 2021-06 | RAN#92 | R5-213934 | 1135 | 1 | F | Annex F for EN-DC and SA FR2 Measurement Accuracy | 16.8.0 |
| 2021-06 | RAN#92 | R5-213935 | 1157 | 1 | F | Align Annex H of TS 38.533 to Clause 7 of TS 38.508-1 | 16.8.0 |
| 2021-06 | RAN#92 | R5-213936 | 1202 | 1 | F | Correction to AoA Test Setup applicability per permitted test method | 16.8.0 |
| 2021-06 | RAN#92 | R5-213937 | 1204 | 1 | F | Adapt Annex F to indicate applicability of the RRM FR2 TT analysis results | 16.8.0 |
| 2021-06 | RAN#92 | R5-213938 | 1207 | 1 | F | Correction of MTSU and applied TT in Annex F for FR2 inter-freq measurement test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-214009 | 1151 | 1 | F | Addition of 5G-SRVCC RRM TC 6.3.1.6-handover | 16.8.0 |
| 2021-06 | RAN#92 | R5-214010 | 1152 | 1 | F | Addition of 5G-SRVCC RRM TC 6.6.5.1-envent triggered reporting non-DRX | 16.8.0 |
| 2021-06 | RAN#92 | R5-214060 | 1182 | 1 | F | Correction to FR1 EN-DC BWP switching test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-214061 | 1198 | 1 | F | Update of CSI reporting time for EN-DC FR1 TC 4.5.3.1 | 16.8.0 |
| 2021-06 | RAN#92 | R5-214062 | 1199 | 1 | F | Update of CSI reporting time for EN-DC FR2 TC 5.5.3.1 | 16.8.0 |
| 2021-06 | RAN#92 | R5-214063 | 1203 | 1 | F | Adapt RRM FR2 test cases to indicate applicability of the TT analysis results | 16.8.0 |
| 2021-06 | RAN#92 | R5-214064 | 1146 | 1 | F | Corrections to NR SA FR1 RRC Re-establishment tests | 16.8.0 |
| 2021-06 | RAN#92 | R5-214065 | 1187 | 1 | F | Correction to FR1 NR SA BWP switch test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-214066 | 1200 | 1 | F | Update of CSI reporting time for SA FR1 TC 6.5.3.1 | 16.8.0 |
| 2021-06 | RAN#92 | R5-214067 | 1201 | 1 | F | Update of CSI reporting time for SA FR2 TC 7.5.3.1 and 7.5.3.2 | 16.8.0 |
| 2021-06 | RAN#92 | R5-214068 | 1188 | 1 | F | Correction to NR inter-RAT measurement test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-214090 | 1197 | 1 | F | Correction to Annex A.9 on the selection of test direction | 16.8.0 |
| 2021-06 | RAN#92 | R5-214091 | 1206 | 1 | F | Extension of the RSRPB Rx Beak Peak Search method | 16.8.0 |
| 2021-06 | RAN#92 | R5-214092 | 1221 | 1 | F | Update on test frequencies selection for inter-frequency test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-214108 | 1216 | 1 | F | Correction of EN-DC FR2 inter-freq measurement test case 5.6.2.1 and 5.6.2.3 | 16.8.0 |
| 2021-06 | RAN#92 | R5-214109 | 1217 | 1 | F | Correction of SA FR2 inter-freq measurement test case 7.6.2.1 and 7.6.2.3 | 16.8.0 |
| 2021-06 | RAN#92 | R5-214111 | 1144 | 1 | F | Correction to Interruptions during measurements on deactivated NR SCC | 16.8.0 |
| 2021-06 | RAN#92 | R5-214113 | 1140 | 1 | F | Correction to FR2 event-triggered reporting of intra frequency cell | 16.8.0 |
| 2021-09 | RAN#93 | R5-214195 | 1232 | - | F | Completion 7.7.2.2 including TT analysis results | 16.9.0 |
| 2021-09 | RAN#93 | R5-214197 | 1234 | - | F | Annex E and F update for FR2 inter-frequency periodic measurements tests | 16.9.0 |
| 2021-09 | RAN#93 | R5-214198 | 1235 | - | F | Completion 7.7.1.1 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214199 | 1236 | - | F | Completion 7.7.2.1 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214200 | 1237 | - | F | Completion 7.7.3.1 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214339 | 1239 | - | F | Correction to 4.5.1.1 message contents and core spec alignment | 16.9.0 |
| 2021-09 | RAN#93 | R5-214342 | 1242 | - | F | Correction to 4.5.1.4 core spec alignment | 16.9.0 |
| 2021-09 | RAN#93 | R5-214343 | 1243 | - | F | Correction to 4.5.1.5 message contents and core spec alignment | 16.9.0 |
| 2021-09 | RAN#93 | R5-214345 | 1245 | - | F | Correction to 4.4.1.1 and core spec alignment | 16.9.0 |
| 2021-09 | RAN#93 | R5-214347 | 1247 | - | F | Editorial corrections and core spec alignment for 5.7.1.1 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214348 | 1248 | - | F | Editorial corrections and core spec alignment for 5.7.2.1 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214349 | 1249 | - | F | Editorial corrections and core spec alignment for 5.7.3.1 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214350 | 1250 | - | F | Core spec alignment for 5.6.1.1 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214351 | 1251 | - | F | Core spec alignment for 5.6.1.2 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214352 | 1252 | - | F | Core spec alignment for 5.6.1.3 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214353 | 1253 | - | F | Core spec alignment for 5.6.1.4 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214354 | 1254 | - | F | Correction to 5.6.2.1 message contents | 16.9.0 |
| 2021-09 | RAN#93 | R5-214355 | 1255 | - | F | Correction to 5.6.2.2 message contents | 16.9.0 |
| 2021-09 | RAN#93 | R5-214356 | 1256 | - | F | Correction to 5.6.2.3 message contents | 16.9.0 |
| 2021-09 | RAN#93 | R5-214357 | 1257 | - | F | Correction to 5.6.2.4 message contents | 16.9.0 |
| 2021-09 | RAN#93 | R5-214358 | 1258 | - | F | Correction to 5.6.2.5 message contents | 16.9.0 |
| 2021-09 | RAN#93 | R5-214359 | 1259 | - | F | Correction to 5.6.2.6 message contents | 16.9.0 |
| 2021-09 | RAN#93 | R5-214360 | 1260 | - | F | Correction to 5.6.2.7 message contents | 16.9.0 |
| 2021-09 | RAN#93 | R5-214361 | 1261 | - | F | Correction to 5.6.2.8 message contents | 16.9.0 |
| 2021-09 | RAN#93 | R5-214363 | 1263 | - | F | Correction to 6.5.1.1 message contents and core spec alignment | 16.9.0 |
| 2021-09 | RAN#93 | R5-214365 | 1265 | - | F | Correction to 6.5.1.5 message contents and core spec alignment | 16.9.0 |
| 2021-09 | RAN#93 | R5-214367 | 1267 | - | F | Correction to 6.3.1.6 and core spec alignment | 16.9.0 |
| 2021-09 | RAN#93 | R5-214368 | 1268 | - | F | Correction to 6.5.2.1 and core spec alignment | 16.9.0 |
| 2021-09 | RAN#93 | R5-214370 | 1270 | - | F | Correction to 6.7.5.1 and core spec alignment | 16.9.0 |
| 2021-09 | RAN#93 | R5-214371 | 1271 | - | F | Correction to 6.7.6.1 and core spec alignment | 16.9.0 |
| 2021-09 | RAN#93 | R5-214372 | 1272 | - | F | Correction to 6.7.7.1 and core spec alignment | 16.9.0 |
| 2021-09 | RAN#93 | R5-214373 | 1273 | - | F | Core spec alignment for 7.6.1.1 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214374 | 1274 | - | F | Core spec alignment for 7.6.1.2 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214375 | 1275 | - | F | Core spec alignment for 7.6.1.3 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214376 | 1276 | - | F | Core spec alignment for 7.6.1.4 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214378 | 1278 | - | F | Correction to 8.3.1.1 and core spec alignment | 16.9.0 |
| 2021-09 | RAN#93 | R5-214395 | 1280 | - | F | Addition of FR1 mobility enhancement TC 6.3.1.9-Intra-band inter-frequency sync DAPS HO in SA for FR1 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214407 | 1281 | - | F | Clarification of test procedure for 4.3.2.2.1 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214408 | 1282 | - | F | Corrections to 4.7.1.x.y SS-RSRP test cases | 16.9.0 |
| 2021-09 | RAN#93 | R5-214409 | 1283 | - | F | Corrections to 4.7.2.x SS-RSRQ test cases | 16.9.0 |
| 2021-09 | RAN#93 | R5-214410 | 1284 | - | F | Corrections to 4.7.3.x SS-SINR test cases | 16.9.0 |
| 2021-09 | RAN#93 | R5-214411 | 1285 | - | F | Corrections to 4.5.1.6 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214412 | 1286 | - | F | Corrections to 4.5.1.8 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214413 | 1287 | - | F | Core spec alignment of EN-DC FR2 PRACH | 16.9.0 |
| 2021-09 | RAN#93 | R5-214414 | 1288 | - | F | Corrections to 6.5.1.2 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214415 | 1289 | - | F | Corrections to 6.5.1.4 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214416 | 1290 | - | F | Corrections to 6.7.1.x.y SS-RSRP test cases | 16.9.0 |
| 2021-09 | RAN#93 | R5-214417 | 1291 | - | F | Corrections to 6.7.2.x SS-RSRQ test cases | 16.9.0 |
| 2021-09 | RAN#93 | R5-214418 | 1292 | - | F | Corrections to 6.7.3.x SS-SINR test cases | 16.9.0 |
| 2021-09 | RAN#93 | R5-214419 | 1293 | - | F | Corrections to 6.5.1.6 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214420 | 1294 | - | F | Corrections to 6.5.1.8 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214421 | 1295 | - | F | Align 6.3.2.2.x to core spec | 16.9.0 |
| 2021-09 | RAN#93 | R5-214422 | 1296 | - | F | Align 6.4.3.1 to core spec | 16.9.0 |
| 2021-09 | RAN#93 | R5-214423 | 1297 | - | F | Align 6.6.2.x to core spec | 16.9.0 |
| 2021-09 | RAN#93 | R5-214424 | 1298 | - | F | Align 6.6.3.x to core spec | 16.9.0 |
| 2021-09 | RAN#93 | R5-214426 | 1300 | - | F | Correction to Table H.3.5-9 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214427 | 1301 | - | F | Clarification CSI-ReportConfig from Annex H | 16.9.0 |
| 2021-09 | RAN#93 | R5-214519 | 1302 | - | F | Addition of FR1 mobility enhancement TC 6.3.1.10-Intra-band inter-frequency asynchronous DAPS HO in SA for FR1 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214520 | 1303 | - | F | Addition of FR1 mobility enhancement TC 6.3.1.11-Inter-band inter-frequency sync DAPS HO in SA for FR1 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214521 | 1304 | - | F | Addition of FR1 mobility enhancement TC 6.3.1.12-Inter-band inter-frequency asynchronousDAPS HO in SA for FR1 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214522 | 1305 | - | F | Update of applicability for RLM TC 4.6.1.3 and 4.6.1.6 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214525 | 1308 | - | F | Update of applicability for RLM TC 6.6.1.3 and 6.6.1.6 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214526 | 1309 | - | F | Correction of cell configuration for SA FR1 TC 6.3.2.1.2 and 6.3.2.1.3 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214527 | 1310 | - | F | Correction of specific message content for SA FR2 TC 7.6.1.4 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214528 | 1311 | - | F | Correction of cell configuration for SA FR2 TC 7.3.2.1.1, 7.3.2.1.2 and 7.3.2.1.3 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214529 | 1312 | - | F | Update of TCI configuration for SA FR2 TC 7.6.2.1 and 7.6.2.3 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214530 | 1313 | - | F | Correction of non-existent config for SA FR2 TC 7.7.1.3.1 | 16.9.0 |
| 2021-09 | RAN#93 | R5-214681 | 1319 | - | F | Core alignment for DRX configuration | 16.9.0 |
| 2021-09 | RAN#93 | R5-214682 | 1320 | - | F | Correction to FR2 event-triggered reporting in DRX test cases | 16.9.0 |
| 2021-09 | RAN#93 | R5-214685 | 1323 | - | F | Correction to Inter-RAT SFTD measurement delay and event triggered reporting tests | 16.9.0 |
| 2021-09 | RAN#93 | R5-214687 | 1325 | - | F | Correction to 4.5.7.1EN-DC FR1 addition and release delay of known PSCell | 16.9.0 |
| 2021-09 | RAN#93 | R5-214688 | 1326 | - | F | Correction to NR SA FR1 - E-UTRAN event-triggered reporting tests | 16.9.0 |
| 2021-09 | RAN#93 | R5-214689 | 1327 | - | F | Correction to the number of entries in the measObjectToAddModList | 16.9.0 |
| 2021-09 | RAN#93 | R5-214693 | 1331 | - | F | Clean up on editor notes for FR2 test cases | 16.9.0 |
| 2021-09 | RAN#93 | R5-214711 | 1335 | - | F | Add minimum conformance requirements for DAPS handover | 16.9.0 |
| 2021-09 | RAN#93 | R5-214832 | 1337 | - | F | Correction to 5G-SRVCC RRM TC 6.3.1.6-handover | 16.9.0 |
| 2021-09 | RAN#93 | R5-214833 | 1338 | - | F | Correction to 5G-SRVCC RRM TC 6.6.5.1-envent triggered reporting non-DRX | 16.9.0 |
| 2021-09 | RAN#93 | R5-214921 | 1340 | - | F | Alignment HO test case 6.3.1.2 with core requirements | 16.9.0 |
| 2021-09 | RAN#93 | R5-214922 | 1341 | - | F | Correction re-establishment test cases 6.3.2.1.x | 16.9.0 |
| 2021-09 | RAN#93 | R5-214923 | 1342 | - | F | Remove gapUE and gapFR1 from iRAT test cases | 16.9.0 |
| 2021-09 | RAN#93 | R5-214924 | 1343 | - | F | Annex A.6.1 for iRAT test cases | 16.9.0 |
| 2021-09 | RAN#93 | R5-214948 | 1344 | - | F | Addition of FR-1 NSA CLI Measurement test cases | 16.9.0 |
| 2021-09 | RAN#93 | R5-214983 | 1349 | - | F | Correction to FR1 EN-DC TCs-RLM | 16.9.0 |
| 2021-09 | RAN#93 | R5-214985 | 1351 | - | F | Correction to FR2 EN-DC TCs-RLM | 16.9.0 |
| 2021-09 | RAN#93 | R5-214986 | 1352 | - | F | Correction to FR2 EN-DC TCs-SCell activation | 16.9.0 |
| 2021-09 | RAN#93 | R5-214987 | 1353 | - | F | Correction to FR1 NR SA TC 6.5.2.1-SCell interruption | 16.9.0 |
| 2021-09 | RAN#93 | R5-214989 | 1355 | - | F | Correction to FR1 NR SA TCs-cell reselection | 16.9.0 |
| 2021-09 | RAN#93 | R5-214991 | 1357 | - | F | Correction to FR1 NR SA TCs-RLM | 16.9.0 |
| 2021-09 | RAN#93 | R5-214993 | 1359 | - | F | Correction to FR2 NR SA TCs-cell reselection | 16.9.0 |
| 2021-09 | RAN#93 | R5-214994 | 1360 | - | F | Correction to FR2 NR SA TCs-SCell activation | 16.9.0 |
| 2021-09 | RAN#93 | R5-214996 | 1362 | - | F | Correction to LTE SA TCs-cell reselection | 16.9.0 |
| 2021-09 | RAN#93 | R5-214997 | 1363 | - | F | Correction to LTE SA TCs-inter-RAT delay | 16.9.0 |
| 2021-09 | RAN#93 | R5-214999 | 1365 | - | F | Correction to cell mapping for CA TCs | 16.9.0 |
| 2021-09 | RAN#93 | R5-215005 | 1367 | - | F | Addition of minimum requirements for FR1 CHO | 16.9.0 |
| 2021-09 | RAN#93 | R5-215006 | 1368 | - | F | Addition of minimum requirements for FR2 CHO | 16.9.0 |
| 2021-09 | RAN#93 | R5-215007 | 1369 | - | F | Addition of NR Mob_Enh RRM TC 6.3.1.7-intra freq sync DAPS HO | 16.9.0 |
| 2021-09 | RAN#93 | R5-215008 | 1370 | - | F | Addition of NR Mob_Enh RRM TC 6.3.1.8-intra freq async DAPS HO | 16.9.0 |
| 2021-09 | RAN#93 | R5-215009 | 1371 | - | F | Addition of NR Mob_Enh RRM TC 6.3.3.1-intra freq CHO | 16.9.0 |
| 2021-09 | RAN#93 | R5-215010 | 1372 | - | F | Addition of NR Mob_Enh RRM TC 6.3.3.2-inter freq CHO | 16.9.0 |
| 2021-09 | RAN#93 | R5-215011 | 1373 | - | F | Addition of NR Mob_Enh RRM TC 7.3.1.4-inter band sync DAPS HO | 16.9.0 |
| 2021-09 | RAN#93 | R5-215012 | 1374 | - | F | Addition of NR Mob_Enh RRM TC 7.3.1.5-inter band async DAPS HO | 16.9.0 |
| 2021-09 | RAN#93 | R5-215013 | 1375 | - | F | Addition of NR Mob_Enh RRM TC 7.3.3.1-intra freq CHO | 16.9.0 |
| 2021-09 | RAN#93 | R5-215014 | 1376 | - | F | Addition of NR Mob_Enh RRM TC 7.3.3.2-inter freq CHO | 16.9.0 |
| 2021-09 | RAN#93 | R5-215020 | 1378 | - | F | Addition of minimum requirements for inter-freq relaxed measurement | 16.9.0 |
| 2021-09 | RAN#93 | R5-215022 | 1380 | - | F | Addition of minimum requirements for intra-freq relaxed measurement | 16.9.0 |
| 2021-09 | RAN#93 | R5-215023 | 1381 | - | F | Addition of NR PS RRM TC 6.1.1.3 - intra-freq cell reselection low mobility | 16.9.0 |
| 2021-09 | RAN#93 | R5-215024 | 1382 | - | F | Addition of NR PS RRM TC 6.1.1.4 - intra-freq cell reselection non-cell-edge | 16.9.0 |
| 2021-09 | RAN#93 | R5-215025 | 1383 | - | F | Addition of NR PS RRM TC 6.1.1.5 - inter-freq cell reselection low mobility | 16.9.0 |
| 2021-09 | RAN#93 | R5-215026 | 1384 | - | F | Addition of NR PS RRM TC 6.1.1.6 - inter-freq cell reselection non-cell-edge | 16.9.0 |
| 2021-09 | RAN#93 | R5-215036 | 1388 | - | F | Correction to minimum requirements for inter-RAT cell reselection with highSpeedMeasFlag | 16.9.0 |
| 2021-09 | RAN#93 | R5-215037 | 1389 | - | F | Correction to minimum requirements for intra-frequency measurement with highSpeedMeasFlag | 16.9.0 |
| 2021-09 | RAN#93 | R5-215038 | 1390 | - | F | Correction to NR HST RRM TC 6.1.1.7-HST intra-freq cell reselection | 16.9.0 |
| 2021-09 | RAN#93 | R5-215039 | 1391 | - | F | Addition of cell mapping for NR HST RRM TCs | 16.9.0 |
| 2021-09 | RAN#93 | R5-215243 | 1398 | - | F | Update of test case 6.6.4.5 for R16 NR HST | 16.9.0 |
| 2021-09 | RAN#93 | R5-215244 | 1399 | - | F | Editorial change of RRM test case 6.6.1.7 | 16.9.0 |
| 2021-09 | RAN#93 | R5-215352 | 1401 | - | F | Correction to test frequeny selection for intra-band EN-DC | 16.9.0 |
| 2021-09 | RAN#93 | R5-215354 | 1403 | - | F | Correction to test requirement for 8.4.2.4 and error in writing for 8.4.2.x | 16.9.0 |
| 2021-09 | RAN#93 | R5-215355 | 1404 | - | F | Correction to DRX configuration for eliminating overlap between DRX and SMTC | 16.9.0 |
| 2021-09 | RAN#93 | R5-215360 | 1405 | - | F | Correction to test case 6.3.1.1 and 6.3.1.3 | 16.9.0 |
| 2021-09 | RAN#93 | R5-215365 | 1406 | - | F | Core spec alignment to add CCR configuration for EN-DC PRACH | 16.9.0 |
| 2021-09 | RAN#93 | R5-215366 | 1407 | - | F | Core spec alignment to add CCR configuration for EN-DC Timing | 16.9.0 |
| 2021-09 | RAN#93 | R5-215367 | 1408 | - | F | Core spec alignment to add CCR configuration for EN-DC event triggered | 16.9.0 |
| 2021-09 | RAN#93 | R5-215368 | 1409 | - | F | Addition of minimum conformance requirements of cell re-selection with relaxed measurement criterion in FR2 | 16.9.0 |
| 2021-09 | RAN#93 | R5-215392 | 1410 | - | F | Add test case 4.3.2.2.3 for EN-DC FR1 2-step PRACH | 16.9.0 |
| 2021-09 | RAN#93 | R5-215393 | 1411 | - | F | Add test case 4.3.2.2.4 for EN-DC FR1 2-step PRACH | 16.9.0 |
| 2021-09 | RAN#93 | R5-215394 | 1412 | - | F | Add test case 5.3.2.2.3 for EN-DC FR2 2-step PRACH | 16.9.0 |
| 2021-09 | RAN#93 | R5-215395 | 1413 | - | F | Add test case 5.3.2.2.4 for EN-DC FR2 2-step PRACH | 16.9.0 |
| 2021-09 | RAN#93 | R5-215396 | 1414 | - | F | Add test case 6.3.2.2.3 for SA FR1 2-step PRACH | 16.9.0 |
| 2021-09 | RAN#93 | R5-215397 | 1415 | - | F | Add test case 6.3.2.2.4 for SA FR1 2-step PRACH | 16.9.0 |
| 2021-09 | RAN#93 | R5-215398 | 1416 | - | F | Add 2-Step PRACH test cases to Annex E | 16.9.0 |
| 2021-09 | RAN#93 | R5-215412 | 1417 | - | F | Addition of cell configuration for RRM HST test cases in Annex E | 16.9.0 |
| 2021-09 | RAN#93 | R5-215413 | 1418 | - | F | Correction of RRM HST Inter-RAT measurements test case 6.6.3.3 | 16.9.0 |
| 2021-09 | RAN#93 | R5-215414 | 1419 | - | F | Correction of RRM HST E-UTRA NR FR1 Cell reselection test case 8.2.1.2 | 16.9.0 |
| 2021-09 | RAN#93 | R5-215415 | 1420 | - | F | Correction of RRM HST E-UTRA NR Inter-RAT event triggered reporting test case 8.4.2.9 | 16.9.0 |
| 2021-09 | RAN#93 | R5-215420 | 1425 | - | F | Correction of RRM EN-DC FR2 CSI-RS-based L1-RSRP measurement in DRX test case 5.6.3.4 | 16.9.0 |
| 2021-09 | RAN#93 | R5-215425 | 1430 | - | F | Correction of RRM EN-DC FR2-FR2 event-triggered reporting in DRX test case 5.6.2.2 including Test Tolerance | 16.9.0 |
| 2021-09 | RAN#93 | R5-215426 | 1431 | - | F | Correction of RRM EN-DC FR2-FR2 event-triggered reporting in DRX with SSB time index detection test case 5.6.2.4 including Test Tolerance | 16.9.0 |
| 2021-09 | RAN#93 | R5-215427 | 1432 | - | F | Correction of RRM SA FR2-FR2 event-triggered reporting in DRX test case 7.6.2.2 including Test Tolerance | 16.9.0 |
| 2021-09 | RAN#93 | R5-215428 | 1433 | - | F | Correction of RRM SA FR2-FR2 event-triggered reporting in DRX with SSB time index detection test case 7.6.2.4 including Test Tolerance | 16.9.0 |
| 2021-09 | RAN#93 | R5-215429 | 1434 | - | F | Editorial correction of RRM FR2 EN-DC event triggered measurement test cases | 16.9.0 |
| 2021-09 | RAN#93 | R5-215430 | 1435 | - | F | Editorial correction of RRM FR2 SA event triggered measurement test cases | 16.9.0 |
| 2021-09 | RAN#93 | R5-215431 | 1436 | - | F | Correction of Measurement Uncertainty and Test Tolerance in Annex F for RRM test cases | 16.9.0 |
| 2021-09 | RAN#93 | R5-215436 | 1437 | - | F | Add MsgA configuration to the TS 38.533 annexes | 16.9.0 |
| 2021-09 | RAN#93 | R5-215614 | 1438 | - | F | Update on 6.3.1.1 to be aligned with TS 38.133 | 16.9.0 |
| 2021-09 | RAN#93 | R5-215620 | 1439 | - | F | MTSU and TT mapping related to Max Device Size in TS 38.533 | 16.9.0 |
| 2021-09 | RAN#93 | R5-215622 | 1441 | - | F | Updates on 5.6.1.3 and 5.6.1.4 CSI-RS RLM test cases test applicability | 16.9.0 |
| 2021-09 | RAN#93 | R5-215624 | 1443 | - | F | Correction in 5.6.1.1, 5.6.1.3 test procedure to configure iterations | 16.9.0 |
| 2021-09 | RAN#93 | R5-215625 | 1444 | - | F | Correction in 5.6.2.1, 5.6.2.2, 5.6.2.3, 5.6.2.4 test procedure to configure iterations | 16.9.0 |
| 2021-09 | RAN#93 | R5-215655 | 1451 | - | F | Update to applicability statement to include gap pattern id 13 for applicable NSA event triggered test cases | 16.9.0 |
| 2021-09 | RAN#93 | R5-215656 | 1452 | - | F | TRS configuration update to NSA FR1 TC 6.5.4.1.1 | 16.9.0 |
| 2021-09 | RAN#93 | R5-215903 | 1238 | 1 | F | Change to EN-DC L1-RSRP test cases to add evaluation rules | 16.9.0 |
| 2021-09 | RAN#93 | R5-215904 | 1240 | 1 | F | Correction to 4.5.1.2 core spec alignment | 16.9.0 |
| 2021-09 | RAN#93 | R5-215905 | 1241 | 1 | F | Correction to 4.5.1.3 message contents and core spec alignment | 16.9.0 |
| 2021-09 | RAN#93 | R5-215906 | 1244 | 1 | F | Correction to 4.5.1.7 message contents and core spec alignment | 16.9.0 |
| 2021-09 | RAN#93 | R5-215907 | 1329 | 1 | F | Correction to FR1 and FR2 event-triggered reporting with gap tests | 16.9.0 |
| 2021-09 | RAN#93 | R5-215908 | 1262 | 1 | F | Change to SA L1-RSRP test cases to add evaluation rules | 16.9.0 |
| 2021-09 | RAN#93 | R5-215909 | 1264 | 1 | F | Correction to 6.5.1.3 message contents and core spec alignment | 16.9.0 |
| 2021-09 | RAN#93 | R5-215910 | 1266 | 1 | F | Correction to 6.5.1.7 message contents and core spec alignment | 16.9.0 |
| 2021-09 | RAN#93 | R5-215911 | 1356 | 1 | F | Correction to FR1 NR SA TCs-inter-RAT accuracy | 16.9.0 |
| 2021-09 | RAN#93 | R5-215912 | 1402 | 1 | F | Correction to test procedure for 6.1.2.2 IRAT ReSelection | 16.9.0 |
| 2021-09 | RAN#93 | R5-215913 | 1277 | 1 | F | Change title of iRAT test cases for clarity | 16.9.0 |
| 2021-09 | RAN#93 | R5-215914 | 1361 | 1 | F | Correction to LTE SA TC 8.5.1.1-SFTD accuracy | 16.9.0 |
| 2021-09 | RAN#93 | R5-215915 | 1279 | 1 | F | Change title of iRAT test cases for clarity - Annexes | 16.9.0 |
| 2021-09 | RAN#93 | R5-215916 | 1333 | 1 | F | Update to Annex H.3.4 | 16.9.0 |
| 2021-09 | RAN#93 | R5-215917 | 1366 | 1 | F | Correction to default configuration-Annex H | 16.9.0 |
| 2021-09 | RAN#93 | R5-215918 | 1421 | 1 | F | Correction of CSI-ReportConfig in Annex H | 16.9.0 |
| 2021-09 | RAN#93 | R5-215932 | 1377 | 1 | F | Addition of cell mapping for Mob_Enh RRM TCs | 16.9.0 |
| 2021-09 | RAN#93 | R5-215938 | 1316 | 1 | F | Addition of NR SA FR2-FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | 16.9.0 |
| 2021-09 | RAN#93 | R5-215939 | 1385 | 1 | F | Addition of cell mapping for NR PS RRM TCs | 16.9.0 |
| 2021-09 | RAN#93 | R5-215940 | 1392 | 1 | F | Addition of minimum conformance requirements of inter-RAT cell re-selection with relaxed measurement criterion | 16.9.0 |
| 2021-09 | RAN#93 | R5-215941 | 1395 | 1 | F | Update of Annex E and Annex F for test cases with relaxed measurement criterion | 16.9.0 |
| 2021-09 | RAN#93 | R5-215948 | 1386 | 1 | F | Addition of NR HST RRM TC 4.6.1.7-intra-freq DRX highSpeedMeasFlag | 16.9.0 |
| 2021-09 | RAN#93 | R5-215949 | 1387 | 1 | F | Addition of NR HST RRM TC 6.1.2.5-intra-freq cell reselection highSpeedMeasFlag | 16.9.0 |
| 2021-09 | RAN#93 | R5-216042 | 1328 | 1 | F | Correction to FR1 Beam Failure Detection and Link Recovery tests | 16.9.0 |
| 2021-09 | RAN#93 | R5-216043 | 1339 | 1 | F | Correction message contents 4.3.2.2.2 | 16.9.0 |
| 2021-09 | RAN#93 | R5-216044 | 1345 | 1 | F | Update to applicability of test cases on CSI-RS based RLM | 16.9.0 |
| 2021-09 | RAN#93 | R5-216045 | 1445 | 1 | F | Updates to DCI based BWP switch NSA FR1 TC 4.5.6.1.1 | 16.9.0 |
| 2021-09 | RAN#93 | R5-216046 | 1446 | 1 | F | Updates to DCI based BWP switch NSA FR1 2DLCA TC 4.5.6.1.2 | 16.9.0 |
| 2021-09 | RAN#93 | R5-216047 | 1447 | 1 | F | Updates to RRC based BWP switch NSA FR1 TC 4.5.6.2.1 | 16.9.0 |
| 2021-09 | RAN#93 | R5-216048 | 1228 | 1 | F | Completion 5.7.1.2 including TT analysis results | 16.9.0 |
| 2021-09 | RAN#93 | R5-216049 | 1229 | 1 | F | Completion 5.7.2.2 including TT analysis results | 16.9.0 |
| 2021-09 | RAN#93 | R5-216050 | 1230 | 1 | F | Completion 5.7.3.2 including TT analysis results | 16.9.0 |
| 2021-09 | RAN#93 | R5-216051 | 1322 | 1 | F | Correction to 6.5.3.1 NR SA FR1 SCell activation and deactivation of known SCell | 16.9.0 |
| 2021-09 | RAN#93 | R5-216052 | 1231 | 1 | F | Completion 7.7.1.2 including TT analysis results | 16.9.0 |
| 2021-09 | RAN#93 | R5-216053 | 1233 | 1 | F | Completion 7.7.3.2 including TT analysis results | 16.9.0 |
| 2021-09 | RAN#93 | R5-216054 | 1346 | 1 | F | Update to applicability of test cases requiring gap pattern ID 4 | 16.9.0 |
| 2021-09 | RAN#93 | R5-216055 | 1299 | 1 | F | Completion Annex C.2.3 | 16.9.0 |
| 2021-09 | RAN#93 | R5-216098 | 1336 | 1 | F | Correction to EN-DC FR2 interruptions at transitions between active and non-active during DRX | 16.9.0 |
| 2021-09 | RAN#93 | R5-216099 | 1324 | 1 | F | Correction to 6.5.2.1 NR SA FR1 interruptions during measurements on deactivated NR SCC | 16.9.0 |
| 2021-09 | RAN#93 | R5-216100 | 1330 | 1 | F | Correction to the propagation condition of NR cell for Inter RAT test cases | 16.9.0 |
| 2021-09 | RAN#93 | R5-216101 | 1318 | 1 | F | Addition of BWP definition for FR2 SSB SCS240kHz | 16.9.0 |
| 2021-09 | RAN#93 | R5-216123 | 1448 | 1 | F | Updates to DCI based BWP switch SA FR1 2DLCA TC 6.5.6.1.1 | 16.9.0 |
| 2021-09 | RAN#93 | R5-216124 | 1449 | 1 | F | Updates to DCI based BWP switch SA FR1 TC 6.5.6.1.2 | 16.9.0 |
| 2021-09 | RAN#93 | R5-216125 | 1450 | 1 | F | Updates to RRC based BWP switch SA FR1 TC 6.5.6.2.1 | 16.9.0 |
| 2021-09 | RAN#93 | R5-216132 | 1347 | 1 | F | Correction to FR1 EN-DC TC 4.5.7.1-PSCell addition | 16.9.0 |
| 2021-09 | RAN#93 | R5-216133 | 1348 | 1 | F | Correction to FR1 EN-DC TCs-BWP switching | 16.9.0 |
| 2021-09 | RAN#93 | R5-216134 | 1350 | 1 | F | Correction to FR1 EN-DC TCs-SCell activation | 16.9.0 |
| 2021-09 | RAN#93 | R5-216135 | 1354 | 1 | F | Correction to FR1 NR SA TCs-BWP switching | 16.9.0 |
| 2021-09 | RAN#93 | R5-216136 | 1358 | 1 | F | Correction to FR1 NR SA TCs-SCell activation | 16.9.0 |
| 2021-09 | RAN#93 | R5-216137 | 1454 | 1 | F | Correction to UL BWP configuration for SA FR1 TC 6.5.2.1 | 16.9.0 |
| 2021-09 | RAN#93 | R5-216138 | 1364 | 1 | F | Correction to LTE SA TCs-SFTD delay | 16.9.0 |
| 2021-09 | RAN#93 | R5-216143 | 1393 | 1 | F | Addition of 6.1.2.3 inter-RAT cell re-selection with relaxed measurement with low mobility | 16.9.0 |
| 2021-09 | RAN#93 | R5-216144 | 1394 | 1 | F | Addition of 6.1.2.4 inter-RAT cell re-selection with relaxed measurement with not at cell edge | 16.9.0 |
| 2021-09 | RAN#93 | R5-216145 | 1422 | 1 | F | Correction of RRM EN-DC FR2 SSB-based L1-RSRP measurement in non-DRX test case 5.6.3.1 including Test Tolerance | 16.9.0 |
| 2021-09 | RAN#93 | R5-216146 | 1423 | 1 | F | Correction of RRM EN-DC FR2 SSB-based L1-RSRP measurement in DRX test case 5.6.3.2 including Test Tolerance | 16.9.0 |
| 2021-09 | RAN#93 | R5-216147 | 1424 | 1 | F | Correction of RRM EN-DC FR2 CSI-RS-based L1-RSRP measurement in non-DRX test case 5.6.3.3 | 16.9.0 |
| 2021-09 | RAN#93 | R5-216148 | 1426 | 1 | F | Correction of RRM NR SA FR2 SSB-based L1-RSRP measurement in non-DRX test case 7.6.3.1 including Test Tolerance | 16.9.0 |
| 2021-09 | RAN#93 | R5-216149 | 1427 | 1 | F | Correction of RRM NR SA FR2 SSB-based L1-RSRP measurement in DRX test case 7.6.3.2 including Test Tolerance | 16.9.0 |
| 2021-09 | RAN#93 | R5-216360 | 1428 | 1 | F | Correction of RRM NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX test case 7.6.3.3 | 16.9.0 |
| 2021-09 | RAN#93 | R5-216361 | 1429 | 1 | F | Correction of RRM NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX test case 7.6.3.4 | 16.9.0 |
| 2021-09 | RAN#93 | R5-216364 | 1314 | 1 | F | Addition of NR SA FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | 16.9.0 |
| 2021-09 | RAN#93 | R5-216365 | 1315 | 1 | F | Addition of NR SA FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | 16.9.0 |
| 2021-09 | RAN#93 | R5-216366 | 1317 | 1 | F | Addition of NR SA FR2-FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | 16.9.0 |
| 2021-09 | RAN#93 | R5-215968 | 1397 | 1 | B | Introduction of n24 | 17.0.0 |
| 2021-12 | RAN#94 | R5-216602 | 1455 | - | F | Update of TC 6.7.1.1.2 | 17.1.0 |
| 2021-12 | RAN#94 | R5-216839 | 1458 | - | F | Correction to CSI-RS based beam failure detection test cases | 17.1.0 |
| 2021-12 | RAN#94 | R5-216840 | 1459 | - | F | Correction to Initial DL and UL BWP for test case 6.3.2.1.2 | 17.1.0 |
| 2021-12 | RAN#94 | R5-216841 | 1460 | - | F | Correction to SRS configuration for FR1 timing test cases | 17.1.0 |
| 2021-12 | RAN#94 | R5-216842 | 1461 | - | F | Correction to SRS configuration for FR2 timing test cases | 17.1.0 |
| 2021-12 | RAN#94 | R5-216843 | 1462 | - | F | Correction to FR1 CA RRM tests for generic channel BW configurations | 17.1.0 |
| 2021-12 | RAN#94 | R5-216844 | 1463 | - | F | Correction to FR1 Active BWP Switch Delay TCs | 17.1.0 |
| 2021-12 | RAN#94 | R5-216845 | 1464 | - | F | Correction to FR1 Redirection test cases | 17.1.0 |
| 2021-12 | RAN#94 | R5-216846 | 1465 | - | F | Correction to message exceptions for CSI-RS based BFR tests | 17.1.0 |
| 2021-12 | RAN#94 | R5-216857 | 1470 | - | F | Correction to FR2 EN-DC RRM TCs - RLM | 17.1.0 |
| 2021-12 | RAN#94 | R5-216860 | 1473 | - | F | Correction to FR2 EN-DC RRM TCs using Table H.3.1-4 for event A4 | 17.1.0 |
| 2021-12 | RAN#94 | R5-216861 | 1474 | - | F | Correction to FR1 NR SA RRM TC 6.1.1.1 - intra-freq reselection | 17.1.0 |
| 2021-12 | RAN#94 | R5-216862 | 1475 | - | F | Correction to FR1 NR SA RRM TC 6.7.6.1 - inter-RAT meas | 17.1.0 |
| 2021-12 | RAN#94 | R5-216864 | 1477 | - | F | Correction to FR1 NR SA RRM TCs 6.5.2.1 - interruption | 17.1.0 |
| 2021-12 | RAN#94 | R5-216866 | 1479 | - | F | Correction to FR2 NR SA RRM TCs using Table H.3.1-4 for event A4 | 17.1.0 |
| 2021-12 | RAN#94 | R5-216868 | 1481 | - | F | Correction to SSB reference configurations | 17.1.0 |
| 2021-12 | RAN#94 | R5-216869 | 1482 | - | F | Correction to Annex H for RRM TCs | 17.1.0 |
| 2021-12 | RAN#94 | R5-216873 | 1485 | - | F | Correction to Mob_enh RRM TC 6.3.3.1 - FR1 intra CHO | 17.1.0 |
| 2021-12 | RAN#94 | R5-216874 | 1486 | - | F | Correction to Mob_enh RRM TC 6.3.3.2 - FR1 inter CHO | 17.1.0 |
| 2021-12 | RAN#94 | R5-216876 | 1488 | - | F | Correction to Mob_enh RRM TC 7.3.1.5 - inter-FR DAPS async | 17.1.0 |
| 2021-12 | RAN#94 | R5-216886 | 1492 | - | F | Correction to PS RRM TC 6.1.1.3 - intra low mobility | 17.1.0 |
| 2021-12 | RAN#94 | R5-216887 | 1493 | - | F | Correction to PS RRM TC 6.1.1.4 - intra not cell edge | 17.1.0 |
| 2021-12 | RAN#94 | R5-216888 | 1494 | - | F | Correction to PS RRM TC 6.1.1.5 - inter low mobility | 17.1.0 |
| 2021-12 | RAN#94 | R5-216889 | 1495 | - | F | Correction to PS RRM TC 6.1.1.6 - inter not cell edge | 17.1.0 |
| 2021-12 | RAN#94 | R5-216890 | 1496 | - | F | Correction to Annex F for PS RRM TCs | 17.1.0 |
| 2021-12 | RAN#94 | R5-216900 | 1497 | - | F | Correction to SRVCC RRM TC 6.3.1.6 - UTRA handover | 17.1.0 |
| 2021-12 | RAN#94 | R5-216902 | 1499 | - | F | Correction to Annex F for SRVCC RRM TCs | 17.1.0 |
| 2021-12 | RAN#94 | R5-216912 | 1500 | - | F | Correction to HST RRM TC 4.6.1.7 - intra-freq meas | 17.1.0 |
| 2021-12 | RAN#94 | R5-216913 | 1501 | - | F | Correction to HST RRM TC 4.6.4.5 - L1-RSRP | 17.1.0 |
| 2021-12 | RAN#94 | R5-216914 | 1502 | - | F | Correction to HST RRM TC 6.1.2.5 - inter-RAT reselection | 17.1.0 |
| 2021-12 | RAN#94 | R5-216915 | 1503 | - | F | Correction to Annex F for HST RRM TCs | 17.1.0 |
| 2021-12 | RAN#94 | R5-217070 | 1505 | - | F | Correction of message configuration for EN-DC FR1 SFTD TC 4.7.5.1 | 17.1.0 |
| 2021-12 | RAN#94 | R5-217122 | 1510 | - | F | Reinstate wrongly deleted Table H.3.5-4 | 17.1.0 |
| 2021-12 | RAN#94 | R5-217123 | 1511 | - | F | Correct B2 Threshold in H.3.1-4A | 17.1.0 |
| 2021-12 | RAN#94 | R5-217124 | 1512 | - | F | Corrections to 4.3.2.2.1 | 17.1.0 |
| 2021-12 | RAN#94 | R5-217131 | 1515 | - | F | Corrections to L1-RSRP EN-DC test cases | 17.1.0 |
| 2021-12 | RAN#94 | R5-217132 | 1516 | - | F | Corrections to L1-RSRP NR SA test cases | 17.1.0 |
| 2021-12 | RAN#94 | R5-217145 | 1517 | - | F | Correct reference to test frequencies | 17.1.0 |
| 2021-12 | RAN#94 | R5-217203 | 1519 | - | F | Addition of minimum requirements for EN-DC L1-SINR measurement for beam reporting | 17.1.0 |
| 2021-12 | RAN#94 | R5-217207 | 1523 | - | F | Addition of minimum requirements for NR SA FR1 L1-SINR measurement for beam reporting | 17.1.0 |
| 2021-12 | RAN#94 | R5-217211 | 1527 | - | F | Addition of the abbreviations for Rel-16 NR MIMO | 17.1.0 |
| 2021-12 | RAN#94 | R5-217213 | 1529 | - | F | Addition of Annex A.1.4B CSI-IM Reference Measurement Channels | 17.1.0 |
| 2021-12 | RAN#94 | R5-217214 | 1530 | - | F | Addition of Annex H.3.6A RRC messages and IE exceptions for L1-SINR measurement for beam reporting | 17.1.0 |
| 2021-12 | RAN#94 | R5-217228 | 1539 | - | F | Correct of RRM Test Cases for test applicability | 17.1.0 |
| 2021-12 | RAN#94 | R5-217229 | 1540 | - | F | Correct of RRM Test Cases | 17.1.0 |
| 2021-12 | RAN#94 | R5-217255 | 1541 | - | F | Addition of FR-1 NSA CLI accuracy test cases | 17.1.0 |
| 2021-12 | RAN#94 | R5-217258 | 1542 | - | F | Corrections to event-triggered iRAT | 17.1.0 |
| 2021-12 | RAN#94 | R5-217395 | 1545 | - | F | Update of 6.1.2.3 and 6.1.2.4 for MU and TT | 17.1.0 |
| 2021-12 | RAN#94 | R5-217396 | 1546 | - | F | Update of Annex F for test cases with relaxed measurement criterion | 17.1.0 |
| 2021-12 | RAN#94 | R5-217436 | 1550 | - | F | Correction to E-UTRA - NR FR1 cell re-selection to higher priority NR target cell | 17.1.0 |
| 2021-12 | RAN#94 | R5-217438 | 1551 | - | F | Correction to SIB4 exceptions in Annex H | 17.1.0 |
| 2021-12 | RAN#94 | R5-217439 | 1552 | - | F | Correction to FR2 event-triggered reporting with gap test | 17.1.0 |
| 2021-12 | RAN#94 | R5-217441 | 1553 | - | F | Correction to test procedure in 8.2.1.1 | 17.1.0 |
| 2021-12 | RAN#94 | R5-217443 | 1555 | - | F | Correction to test requirement in 8.3.1.1 | 17.1.0 |
| 2021-12 | RAN#94 | R5-217520 | 1559 | - | F | Addition of RMC CORESET to FR2 random access tests | 17.1.0 |
| 2021-12 | RAN#94 | R5-217570 | 1561 | - | F | Correction of RRM HST Inter-RAT measurements test case 6.6.3.3 including Test Tolerance | 17.1.0 |
| 2021-12 | RAN#94 | R5-217572 | 1563 | - | F | Correction of RRM HST E-UTRA NR Inter-RAT event triggered reporting test case 8.4.2.9 including Test Tolerance | 17.1.0 |
| 2021-12 | RAN#94 | R5-217573 | 1564 | - | F | Correction of RRM EN-DC FR2 CSI-RS-based L1-RSRP measurement in non-DRX test case 5.6.3.3 including Test Tolerance | 17.1.0 |
| 2021-12 | RAN#94 | R5-217574 | 1565 | - | F | Correction of RRM EN-DC FR2 CSI-RS-based L1-RSRP measurement in DRX test case 5.6.3.4 including Test Tolerance | 17.1.0 |
| 2021-12 | RAN#94 | R5-217575 | 1566 | - | F | Correction of RRM NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX test case 7.6.3.3 including Test Tolerance | 17.1.0 |
| 2021-12 | RAN#94 | R5-217576 | 1567 | - | F | Correction of RRM NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX test case 7.6.3.4 including Test Tolerance | 17.1.0 |
| 2021-12 | RAN#94 | R5-217577 | 1568 | - | F | Correction of SA FR2 RRC re-establishment test case 7.3.2.1.1 | 17.1.0 |
| 2021-12 | RAN#94 | R5-217580 | 1571 | - | F | Correction of Measurement Uncertainty and Test Tolerance in Annex F for RRM test cases | 17.1.0 |
| 2021-12 | RAN#94 | R5-217585 | 1572 | - | F | Correction of Minimum conformance requirements for FR1 re-establishment test cases | 17.1.0 |
| 2021-12 | RAN#94 | R5-217586 | 1573 | - | F | Correction of Minimum conformance requirements for FR2 re-establishment test cases | 17.1.0 |
| 2021-12 | RAN#94 | R5-217661 | 1574 | - | F | Update on FR2 Radio Link monitoring SSB based RLM test cases | 17.1.0 |
| 2021-12 | RAN#94 | R5-217663 | 1575 | - | F | Update on FR2 radio link monitoring CSI-RS based RLM test cases | 17.1.0 |
| 2021-12 | RAN#94 | R5-217736 | 1576 | - | F | Update to SA Handover delay requirements | 17.1.0 |
| 2021-12 | RAN#94 | R5-218207 | 1577 | - | F | Correct 5.7.3.2 / 7.7.3.2 test case limits based on TT analysis results | 17.1.0 |
| 2021-12 | RAN#94 | R5-218215 | 1457 | 1 | F | Correction to Mob_enh RRM TC 6.3.1.9 - inter-freq sync DAPS HO | 17.1.0 |
| 2021-12 | RAN#94 | R5-218216 | 1491 | 1 | F | Correction to Annex F for Mob_enh RRM TCs | 17.1.0 |
| 2021-12 | RAN#94 | R5-218217 | 1504 | 1 | F | Correction to Mob_enh RRM TC 6.3.1.10 - inter-freq async DAPS HO | 17.1.0 |
| 2021-12 | RAN#94 | R5-218250 | 1469 | 1 | F | Correction to FR1 EN-DC RRM TCs 4.5.2.X - interruption | 17.1.0 |
| 2021-12 | RAN#94 | R5-218251 | 1513 | 1 | F | Corrections to RLM EN-DC test cases | 17.1.0 |
| 2021-12 | RAN#94 | R5-218252 | 1548 | 1 | F | Correction to EN-DC FR1 addition and release delay of known PSCell | 17.1.0 |
| 2021-12 | RAN#94 | R5-218253 | 1471 | 1 | F | Correction to FR2 EN-DC RRM TCs - RRC based | 17.1.0 |
| 2021-12 | RAN#94 | R5-218254 | 1506 | 1 | F | Correction of T2 timer value for SA FR1 TC 6.3.2.1.3 | 17.1.0 |
| 2021-12 | RAN#94 | R5-218255 | 1514 | 1 | F | Corrections to RLM NR SA test cases | 17.1.0 |
| 2021-12 | RAN#94 | R5-218256 | 1478 | 1 | F | Correction to FR2 NR SA RRM TCs - RLM | 17.1.0 |
| 2021-12 | RAN#94 | R5-218257 | 1507 | 1 | F | Correction of RRC re-establishment delay for SA FR2 TC 7.3.2.1.1 | 17.1.0 |
| 2021-12 | RAN#94 | R5-218258 | 1508 | 1 | F | Correction of T2 timer value for SA FR2 TC 7.3.2.1.2 | 17.1.0 |
| 2021-12 | RAN#94 | R5-218259 | 1509 | 1 | F | Correction of T2 timer value for SA FR2 TC 7.3.2.1.3 | 17.1.0 |
| 2021-12 | RAN#94 | R5-218260 | 1569 | 1 | F | Correction of SA FR2 - FR2 RRC re-establishment test case 7.3.2.1.2 | 17.1.0 |
| 2021-12 | RAN#94 | R5-218261 | 1480 | 1 | F | Correction to LTE SA RRM TCs - NR inter-RAT accuracy | 17.1.0 |
| 2021-12 | RAN#94 | R5-218294 | 1544 | 1 | F | Update of Annex for 4.5.8.1 EN-DC FR1 DL interruptions at switching between two uplink carriers | 17.1.0 |
| 2021-12 | RAN#94 | R5-218296 | 1483 | 1 | F | Correction to Mob_enh RRM TC 6.3.1.7 - intra DAPS sync | 17.1.0 |
| 2021-12 | RAN#94 | R5-218297 | 1484 | 1 | F | Correction to Mob_enh RRM TC 6.3.1.8 - intra DAPS async | 17.1.0 |
| 2021-12 | RAN#94 | R5-218298 | 1487 | 1 | F | Correction to Mob_enh RRM TC 7.3.1.4 - inter-FR DAPS sync | 17.1.0 |
| 2021-12 | RAN#94 | R5-218299 | 1490 | 1 | F | Correction to Mob_enh RRM TC 7.3.3.2 - FR2 inter CHO | 17.1.0 |
| 2021-12 | RAN#94 | R5-218313 | 1520 | 1 | F | Addition of 4.6.7.1 EN-DC FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in non-DRX | 17.1.0 |
| 2021-12 | RAN#94 | R5-218314 | 1521 | 1 | F | Addition of 4.6.7.2 EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR measurement in DRX | 17.1.0 |
| 2021-12 | RAN#94 | R5-218315 | 1522 | 1 | F | Addition of 4.6.7.3 EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in DRX | 17.1.0 |
| 2021-12 | RAN#94 | R5-218316 | 1524 | 1 | F | Addition of 6.6.8.1 NR SA FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX | 17.1.0 |
| 2021-12 | RAN#94 | R5-218317 | 1525 | 1 | F | Addition of 6.6.8.2 NR SA FR1 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX | 17.1.0 |
| 2021-12 | RAN#94 | R5-218318 | 1526 | 1 | F | Addition of 6.6.8.3 NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX | 17.1.0 |
| 2021-12 | RAN#94 | R5-218319 | 1528 | 1 | F | Addition of CSI-RS Reference Measurement Channels based on IMR | 17.1.0 |
| 2021-12 | RAN#94 | R5-218320 | 1531 | 1 | F | Addition of 4.5.5.5 EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX | 17.1.0 |
| 2021-12 | RAN#94 | R5-218321 | 1532 | 1 | F | Addition of 4.5.5.6 EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX | 17.1.0 |
| 2021-12 | RAN#94 | R5-218322 | 1533 | 1 | F | Addition of 4.7.7.1 EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy | 17.1.0 |
| 2021-12 | RAN#94 | R5-218323 | 1535 | 1 | F | Addition of 4.7.7.3 EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | 17.1.0 |
| 2021-12 | RAN#94 | R5-218324 | 1536 | 1 | F | Addition of 6.7.9.1 NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy | 17.1.0 |
| 2021-12 | RAN#94 | R5-218325 | 1537 | 1 | F | Addition of 6.7.9.2 NR SA FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy | 17.1.0 |
| 2021-12 | RAN#94 | R5-218326 | 1538 | 1 | F | Addition of 6.7.9.3 NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | 17.1.0 |
| 2021-12 | RAN#94 | R5-218327 | 1547 | 1 | F | Addition of cell configuration MU and TT for Enhancement on MIMO | 17.1.0 |
| 2021-12 | RAN#94 | R5-218336 | 1498 | 1 | F | Correction to SRVCC RRM TC 6.6.5.1 - UTRA meas | 17.1.0 |
| 2021-12 | RAN#94 | R5-218351 | 1562 | 1 | F | Correction of RRM HST E-UTRA NR FR1 Cell reselection test case 8.2.1.2 including Test Tolerance | 17.1.0 |
| 2021-12 | RAN#94 | R5-218352 | 1570 | 1 | F | Correction of Measurement Uncertainty and Test Tolerance in Annex F for RRM HST test cases | 17.1.0 |
| 2021-12 | RAN#94 | R5-218372 | 1543 | 1 | F | Addition of 4.5.8.1 EN-DC FR1 DL interruptions at switching between two uplink carriers | 17.1.0 |
| 2021-12 | RAN#94 | R5-218439 | 1468 | 1 | F | Correction to FR1 EN-DC RRM TCs - RRC based | 17.1.0 |
| 2021-12 | RAN#94 | R5-218440 | 1560 | 1 | F | Test Procedure update for FR1 Event Triggered test cases with CDRX | 17.1.0 |
| 2021-12 | RAN#94 | R5-218441 | 1472 | 1 | F | Correction to FR2 EN-DC RRM TCs 5.5.2.X - interruption | 17.1.0 |
| 2021-12 | RAN#94 | R5-218442 | 1558 | 1 | F | Update to FR2 interruption tests 5.5.2.1 and 5.5.2.2 | 17.1.0 |
| 2021-12 | RAN#94 | R5-218443 | 1456 | 1 | F | Addition of clause 3A.6 for UE with Multiband Capability | 17.1.0 |
| 2021-12 | RAN#94 | R5-218444 | 1556 | 1 | F | Clarification on cl 3A.1.1 test coverage across 5G NR architecture options for RRM | 17.1.0 |
| 2021-12 | RAN#94 | R5-218461 | 1489 | 1 | F | Correction to Mob_enh RRM TC 7.3.3.1 - FR2 intra CHO | 17.1.0 |
| 2021-12 | RAN#94 | R5-218464 | 1534 | 1 | F | Addition of 4.7.7.2 EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy | 17.1.0 |