3GPP TR 38.828 V0.0.1 (2019-04)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Cross Link Interference (CLI) handling and

Remote Interference Management (RIM) for NR;

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<Cross Link Interference (CLI), Remote Interference Management (RIM)>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	4

2	References	4

3	Definitions, symbols and abbreviations	5

3.1	Definitions	5

3.2	Symbols	5

3.3	Abbreviations	5

4	Background	5

4.1	General	5

4.2 	WID description	5

4.2.1	Justification	5

4.2.2	Objective	6

4.3	Dynamic TDD interference scenarios	7

5	Co-existence analysis	7

5.1	Scenarios	7

5.1.1	FR1	7

5.1.2	FR2	7

5.2	Simulation assumptions	7

5.2.1	FR1	7

5.2.1.1 	Network layout model	7

5.2.1.2 	ACIR	7

5.2.1.3 	UE distribution	7

5.2.1.4 	Other simulation parameters	7

5.2.1.5 	Antenna configuration	7

5.2.2	FR2	8

5.2.2.1 	Network layout model	8

5.2.2.2 	ACLR and ACS	8

5.2.2.3 	UE distribution	8

5.2.2.4 	Other simulation parameters	8

5.2.2.5 	Antenna configuration	8

5.2.3	Common assumptions	8

5.2.3.1 	Propagation model	8

5.2.3.2 	LOS model	8

5.2.3.3 	O-to-I penetration loss	8

5.2.3.4 	Transmission power control model	8

5.2.3.5 	Received signal power model	8

5.2.3.6 	Evaluation metric	8

5.2.3.7 	Antenna modelling	8

5.2.3.8 	Simulation description	8

5.3	Simulation results	8

5.3.1	FR1	8

5.3.2	FR2	8

6	Summary and recommendations	9

6.1	FR1	9

6.2	FR2	9

Annex A: Detailed simulation results for non-zero grid shift	9

Annex B: Detailed analysis for zero grid shift	9

Annex C: Change history	9

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

1	Scope

The present document provides the results of co-existence evaluation for CLI and RIM for NR.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-182864: "Revised WID on Cross link Interference (CLI) handling and Remote Interference Management (RIM) for NR".

[3]	3GPP TR 38.802: "Study on new radio access technology Physical layer aspects".

[4]	3GPP TR 38.866: "Study on remote interference management for NR".

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

3.2	Symbols

Void

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

CLI	Cross Link Interference

RIM	Remote Interference Management

4	Background

4.1	General

4.2 	WID description

4.2.1	Justification

As captured in TR38.802NR aims to support duplexing flexibility in both paired and unpaired spectrum.

“NR supports paired and unpaired spectrum and strives to maximize commonality between the technical solutions, allowing FDD operation on a paired spectrum, different transmission directions in either part of a paired spectrum, TDD operation on an unpaired spectrum where the transmission direction of time resources is not dynamically changed, and TDD operation on an unpaired spectrum where the transmission direction of most time resources can be dynamically changing. DL and UL transmission directions at least for data can be dynamically assigned on a per-slot basis at least in a TDM manner. It is noted that transmission directions include all of downlink, uplink, sidelink, and backhaul link. NR supports at least semi-statically assigned DL/UL transmission direction as gNB operation, i.e., the assigned DL/UL transmission direction can be signaled to UE by higher layer signaling.”

Rel-14 NR study showed that duplexing flexibility with cross-link interference mitigation shows better user throughput compared to static UL/DL operation or dynamic UL/DL operation without interference mitigation in indoor hotspot (4GHz and 30GHz) and urban macro scenarios (4GHz and 2GHz). The mitigation techniques include coordinated scheduling scheduling/beamforming, power control, link adaptation, hybrid dynamic/static UL/DL resource assignment.

During Rel-15 NR WI, enablers for basic support of cross-link interference mitigation schemes to support duplexing flexibility for paired and unpaired spectrum were discussed but could not specified as the work has been deprioritized. Mainly, it was agreed to introduce UE-to-UE measurement for CLI, and TRP-to-TRP measurement/coordination techniques were discussed. Rel-15 NR specification supports mechanisms to allow dynamic DL/UL assignments. Yet, any cross-link interference mitigation techniques and coexistence requirements have not been specified and thus the use of dynamic DL/UL assignment operation is considerably restricted.

As observed during study, dynamic DL/UL resource assignments would be more beneficial in indoor hotspot and urban macro scenarios where gNB TX power is rather limited, and ISD is small. During the study, dynamic DL/UL assignment operations offers performance gain in high frequencies such as 4 GHz and 30 GHz. In such frequencies, advanced MIMO techniques can be utilized to mitigate cross-link interference which has not been fully evaluated.

Furthermore, semi-static and/or dynamic DL/UL resource assignments should also consider coexistence issues particularly among different operators where tight coordination are challenged. Coexistence mechanisms among different operators in co-channel and adjacent channels are essential regardless whether each cell operates in semi-static or dynamic DL/UL assignment mechanism. For efficient coexistence, not only coexistence requirements need to be understood but also advanced mechanisms to mitigate interference such as TRP-to-TRP measurement and adaptation based on measurements should be considered.

In commercial TD-LTE network with macro deployment scenario, it is observed in relatively large scale of eNBs that IoT (amounts to above -105dBm, even to -90dBm in some extreme cases) in these eNBs intermittently deteriorated to severely impact the network coverage and connection successful rate. Jointed with several approaches, e.g., IoT statistics from the eNBs in some regions over the forecast of troposphere bending (http://www.dxinfocentre.com/tropo.html) , as well as the symptoms varying with the artificially constructed transmission patterns, it is identified that this kind of IoT degradation is caused by the downlink signal of remote eNB (furthest as 300km away with observed record) as long as the atmospheric conditions favourable for producing troposphere bending of radio waves are available. To mitigate the impact of this kind of remote base station interference as it intermittently happens, but not to scarify the network resources all the time, in the field TD-LTE network, some adaptive mechanisms are introduced, where abnormal IoT enhancement will trigger the victim eNB to transmit a specific signal in a window, each eNB that detected the specific signal in a window will identify itself as the contributor of the deteriorated IoT in some eNB(s), and then it will reconfigure the GP or some other parameters to reduce its weight to the interference. In this framework, the impact of troposphere bending is mitigated to some extension, however, some disadvantages are also apparent in the proprietary implementation, e.g., rely on some static mechanism for detection signal transmission and detection  due to lack inter-vendor inter-eNB coordination, decision making is per individual eNB based implementation, etc.

In NR deployment on lower TDD frequency, the impact of the troposphere bending will continue existing if no special mechanisms are introduced. Though the design of the frame structure in NR has already considered much more flexible GP to leave larger room for avoiding the remote interference, it is necessary to study mechanisms for identifying when or how long will the long enough GP be configured, as well as corresponding gNB’s behaviour and inter-gNB’s coordination procedure.

In the RIM SI, the frameworks for mechanisms for gNBs to start and terminate the transmission/detection of reference signal(s), the functionalities and requirements of the corresponding RS(s) as well as the design of the RS(s), and the backhaul-based coordination mechanisms among gNBs have been studied, where the outcome and conclusions of the SI was captured in TR38.866. It is recommended to specify RIM RS(s) to support identifying remote interference related information, it is also recommended to specify the inter-set RIM backhaul signalling via the core network for backhaul-based solution.

4.2.2	Objective

In the WID on Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR , the detailed objectives for cross-link interference mitigation to support flexible resource adaptation for unpaired NR cells are:

- Specify cross-link interference measurements and reporting at a UE (e.g., CLI-RSSI and/or CLI-RSRP) [RAN1, RAN2, RAN4]

- Perform coexistence study to identify conditions of coexistence among different operators in adjacent channels [RAN4]

  - Target no or very minimal impact on RF requirement

Note: Measurement and coordination mechanisms should be applicable to IAB nodes.

The detailed objectives for remote-interference management are:

- Specify RIM RS resource and configurations, including [RAN1]

  - A basic RIM-RS resource

  - Configuration of RIM-RS and distinguishable RIM RS-1/2 resources, including sequence type, time and frequency transmission pattern

  - Determine gNB set identification information through detection of RIM-RS(s) by implicit or explicit indication. Determine further information that can be carried by the RIM-RS, such as “Ducting phenomenon exists”, “Enough mitigation” & “Not enough mitigation”,   [RAN1, SA5]

- Specify the inter-set RIM backhaul signalling via the core network to convey the messages of “RIM-RS detected” and “RIM-RS disappeared [RAN3]

- Identify corresponding OAM functions to support RIM operation [RAN1, RAN3].

This TR 38.828 focuses on evaluating co-existence of CLI among different operators in adjacent channels and RAN4 targets no or very minimal impact on RF requirement.

4.3	Dynamic TDD interference scenarios

5	Co-existence analysis

5.1	Scenarios

5.1.1	FR1

5.1.2	FR2

5.2	Simulation assumptions

5.2.1	FR1

5.2.1.1 	Network layout model

5.2.1.2 	ACIR

5.2.1.3 	UE distribution

5.2.1.4 	Other simulation parameters

5.2.1.5 	Antenna configuration

5.2.2	FR2

5.2.2.1 	Network layout model

5.2.2.2 	ACLR and ACS

5.2.2.3 	UE distribution

5.2.2.4 	Other simulation parameters

5.2.2.5 	Antenna configuration

5.2.3	Common assumptions

5.2.3.1 	Propagation model

5.2.3.2 	LOS model

5.2.3.3 	O-to-I penetration loss

5.2.3.4 	Transmission power control model

5.2.3.5 	Received signal power model

5.2.3.6 	Evaluation metric

5.2.3.7 	Antenna modelling

5.2.3.8 	Simulation description

5.3	Simulation results

5.3.1	FR1

5.3.2	FR2

6	Summary and recommendations

6.1	FR1

6.2	FR2

Annex A: Detailed simulation results for non-zero grid shift

Annex B: Detailed analysis for zero grid shift

Annex C: Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-04 | 3GPP RAN4 #90BIS | R4-1903037 |  |  |  | TR skeleton for Co-existence study of Cross-Link Interference (CLI) | 0.0.1 |