| 3GPP TR 38.762 V1.0.0 (2025-09) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) dynamic test methodology for FR1 UEs (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	8

4	General	9

4.1	Device types	9

4.2	Testing configurations	9

4.3	Testing bands	9

5	Performance metrics	10

6	Measurement methodology	10

6.1	General	10

6.2	Environmental conditions	10

6.3	Multi-Probe Anechoic Chamber (MPAC)	11

6.3.1	System setup	11

6.3.2	Minimum range length and test zone size	12

6.3.3	Ripple test for quiet zone	12

6.3.4	Calibration procedure	12

6.3.5	Test procedure	12

6.4	UE positioning in MPAC	12

7	Channel models	13

7.1	Dynamic channel model definition	13

7.2	Emulation of base station beamforming configuration	16

8	Channel model validation	17

8.1	General	17

8.2	Validation of the dynamic channel models	17

8.2.1	Measurement Setup	17

8.2.1.1	Network Analyser (VNA) Setup	18

8.2.1.2	Signal Analyser (SAN) Setup	18

8.2.2	Validation of dynamic path loss	18

8.2.2.1	PL Method of Measurement	18

8.2.2.2	PL Measurement Antenna	19

8.2.2.3	PL Measurement Results Analysis	19

8.2.2.4	PL Target Values	20

8.2.3	Validation of Power Delay Profile (PDP)	21

8.2.3.1	General	21

8.2.3.2	PDP Method of Measurement using Frequency-Domain Technique	22

8.2.3.3	PDP Method of Measurement using Time-Domain Technique	22

8.2.3.4	PDP Measurement Antenna	22

8.2.3.5	PDP Measurement Results Analysis using Frequency-Domain Technique	22

8.2.3.6	PDP Measurement Results Analysis using Time-Domain Technique	23

8.2.3.7	PDP Target Values	23

8.2.3.8	Chebyshev Window Function	27

8.2.4	Validation of Doppler/Temporal Correlation	28

8.2.4.1	TCF Method of Measurement	28

8.2.4.2	TCF Measurement Antenna	29

8.2.4.3	TCF Measurement Results Analysis	29

8.2.4.4	TCF Target Values	29

8.2.5	Validation of Spatial Correlation	31

8.2.5.1	SCF Method of Measurement	31

8.2.5.2	SCF Measurement Antenna	33

8.2.5.3	SCF Measurement Results Analysis	33

8.2.5.4	SCF Target Values	33

8.2.6	Validation of Cross-Polarization	35

8.2.6.1	XPO Method of Measurement	35

8.2.6.2	XPO Measurement Antenna	35

8.2.6.3	XPO Measurement Results Analysis	35

8.2.6.4	XPO Target Values	36

8.3	Validation pass/fail criteria	37

8.3.1	General	37

8.3.2	Pass/Fail Criteria of dynamic path loss	37

8.3.3	Pass/Fail Criteria of PDP	37

8.3.4	Pass/Fail Criteria of Doppler/Temporal correlation	38

8.3.5	Pass/Fail Criteria of Spatial correlation	38

8.3.6	Pass/Fail Criteria of Cross-polarization	39

9	gNodeB configurations	39

9.1	General	39

9.2	gNodeB emulator settings	39

9.3	Dynamic link adaptation for MCS, rank and MIMO precoding	45

Annex A: Channel model parameters	46

Annex B: UE coordinate system	59

B.1	Reference coordinate system	59

B.2	DUT positioning guidelines	61

Annex C: Measurement uncertainty	62

C.1	Preliminary MU assessment for MPAC	62

C.2	MU contribution descriptions for MPAC	63

Annex D: Environmental requirements	64

D.1	Ambient temperature	64

D.2	Operating voltage	64

Annex E: Change history	65

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document is a technical report for the Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) dynamic test methodology for NR FR1 UEs developed in the work item TRP_TRS_MIMO_OTA_Ph3.

In Rel-16, Rel-17, and Rel-18, the static MIMO OTA test methodology for NR UEs was developed and enhanced. This TR targets to define dynamic test methodology and radiated metrics for multi-antenna reception performance of NR FR1 UEs.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

-	In the case where the same reference is dated and undated, the specific reference to the document in the test plan shall be considered to determine if the dated and undated version is to be used.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[3]	3GPP TR 37.977: "Verification of radiated multi-antenna reception performance of User Equipment (UE)".

[4]	3GPP TS 38.151: "Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) performance requirements".

[5]	3GPP TR 38.827: "Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)".

[6]	L. Hentilä, P. Kyösti and P. Heino, "Evaluation of beam forming and multi antenna techniques in non-stationary propagation scenarios with HW emulator," 2012 International ITG Workshop on Smart Antennas (WSA), Dresden, 2012, pp. 347-351. Online: https://ieeexplore.ieee.org/document/6181232

[7]	3GPP TR 38.901 V18.0.0 (2024-03): "Study on channel model for frequencies from 0.5 to 100 GHz".

[8]	3GPP TS 38.101-4: "User Equipment (UE) radio transmission and reception; Part 4: Performance requirements".

[9]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[10]	3GPP TS 38.551: “User Equipment (UE) Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) performance; Conformance testing”

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Black-box testing: A testing approach where the DUT's performance is evaluated through radiated measurements without prior knowledge of its internal antenna design.

CTMT: The cumulative distribution function of the Total Measured Throughput, serving as the performance metric for dynamic MIMO OTA tests under link adaptation and dynamic channel models.

FS: UE used in a free space configuration.

Handheld UE: A UE intended to be used in hand held scenario.

Primary mechanical mode: The mode that is most often used for a specific user scenario. Every terminal has at least one primary mechanical mode, if multiple modes are supported, different primary mechanical modes may be applicable for different user scenarios, e.g., different primary mechanical modes for Free Space and Hand phantom usage for the same UE.

Total Measured Throughput: The combination of all measured throughput values recorded during MIMO OTA tests under link adaptation and dynamic channel models, aggregated across all DUT orientations and all rotations for a given channel model.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AoA	Azimuth angle of Arrival

AoD	Azimuth angle of Departure

BS	Base Station

CE	Channel Emulator

CDL	Clustered Delay Line

CDF	Cumulative Distribution Function

CQI	Channel Quality Indicator

CSI-RS	Channel State Information Reference Signal

CTMT	CDF of the Total Measured Throughput

CW	Continuous Wave

DL	Down Link

DML	Data Mode Landscape

DMP	Data Mode Portrait

DMSU	Data Mode Screen Up

DoT	Direction of Travel

DUT	Device Under Test

EUT	Equipment Under Test

FR1	Frequency Range 1

FS	Free Space

gNodeB	Next Generation NodeB (5G)

HARQ	Hybrid Automatic Repeat Request

HPBW	Half Power Bandwidth

LOS	Line of Sight

MCS	Modulation and Coding Scheme

MIMO	Multiple Input Multiple Output

MPAC	Multi-Probe Anechoic Chamber

NLOS	Non-Line of Sight

NR	New Radio

NSA	Non-Standalone, a mode of operation where operation of another radio is assisted with another radio

OTA	Over The Air

PAS	Power Angular Spectrum

PDP	Power Delay Profile

PL	Path Loss

PMI	Precoding Matrix Indicator

RI	Rank Indicator

RMC	Reference Measurement Channel

SAN	Signal Analyser

SCF	Spatial correlation Function

SG	Signal Generator

SIR	Signal to Interference Ratio

SS 	System Simulator

SSS	Secondary Synchronization Signal

TCF	Temporal Correlation Function

TMT	Total Measured Throughput

TP	Throughput

UE	User Equipment

UMa	Urban Macro

UMi	Urban Micro

VNA	Vector Network Analyzer

XPR	Cross-Polarization Ratio

ZoA	Zenith angle of Arrival

ZoD	Zenith angle of Departure

ZSA	Zenith angle Spread of Arrival

ZSD	Zenith angle Spread of Departure

4	General

4.1	Device types

The following device types are within the scope of the study on dynamic MIMO OTA test methodology:

-	Smartphone

-	Other UE types are not precluded for discussion as a second priority

The development of test methodology aspects shall initially focus on the smartphone device type.

4.2	Testing configurations

Free space (FS) testing configuration is the first priority.

4.3	Testing bands

The frequency ranges in which NR can operate are identified as described in Table 4.3-1.

Table 4.3-1: Definition of frequency ranges

| Frequency range designation |  | Corresponding frequency range |
| --- | --- | --- |
| FR1 |  | 410 MHz – 7125 MHz |
| FR2 | FR2-1 | 24250 MHz – 52600 MHz |
|  | FR2-2 | 52600 MHz – 71000 MHz |

The present technical report covers FR1 operating bands.

Operating bands for NR FR1 are defined in Table 5.2-1 in TS 38.101-1 [2].

5	Performance metrics

Let the CDF of the Total Measured Throughput (CTMT) be the figure of merit for the MIMO OTA tests with link adaptation and dynamic channel models. The TP shall be recorded every 200 ms during the emulation of the route separately for each of the M DUT orientations and for each of the L rotations per orientation, defined in Clause B.2. The Total Measured Throughput (TMT) is defined as the combination/concatenation of all measured TP values for each channel model, i.e.,

TMTUMi=TP1,1(t1),..., TP1,1(tN), ...,TPm,l(t1),..., TPm,l(tN),..., TPM,L(t1),..., TPM,L(tN)UMi

and

TMTUMa=TP1,1(t1),..., TP1,1(tN), ...,TPm,l(t1),..., TPm,l(tN),..., TPM,L(t1),..., TPM,L(tN)UMa

with m = 1 to M=4 and l = 1 to L=4.

Subsequently, the TMTs from both channel models are combined into a single one

TMTUMi&UMa=TMTUMi, TMTUMa

Three separate CTMT values shall be determined from the TMT values, i.e., the 10%, 50%, and 90%-ile CDF

-	CTMTUMi&UMa,10%=CDF10%TMTUMi&UMa

-	CTMTUMi&UMa,50%=CDF50%TMTUMi&UMa

-	CTMTUMi&UMa,90%=CDF90%(TMTUMi&UMa)

The final CTMT metric is the weighted average of the three CTMTs, i.e.,

CTMTUMi&UMa=w10CTMTUMi&UMa,10%+w50CTMTUMi&UMa,50%+w90CTMTUMi&UMa,90%

with the weights w10, w50, w90 FFS.

6	Measurement methodology

6.1	General

This clause describes basic aspects of the dynamic MIMO OTA measurement methodology, including environmental conditions, the multi-probe anechoic chamber (MPAC) system, and UE positioning.

6.2	Environmental conditions

The test cases in this document shall utilize the UE noise limit condition, i.e., UE throughput is characterized as a function of signal power incident to the DUT antennas.

Here, the signal power takes the full UMa/UMi path loss model into account, i.e., the path loss is modelled per Table 7.4.1-1 of [7]. The normalized signal power with gNB antenna effect would therefore be modelled as illustrated in Figure 6.2-1 for the UMa route/model and in Figure 6.2-2 for the UMi route/model.

Figure 6.2-1: Signal Power Profile with gNB antenna effect for UE Noise-Limited Environment using UMa Route with full UMa path loss model from Table 7.4.1-1 of [7]

Figure 6.2-2: Signal Power Profile with gNB antenna effect for UE Noise-Limited Environment using UMi Route with full UMi path loss model from Table 7.4.1-1 of [7]

6.3	Multi-Probe Anechoic Chamber (MPAC)

6.3.1	System setup

The Multi-Probe Anechoic Chamber is based on the anechoic boundary array concept used for LTE [3] and NR FR1 MIMO with static channel models [4].

Figure 6.3.1-1 illustrates a typical 16 element boundary array configuration with 32 channels of spatial channel emulation; the intention is to apply fading on the DL signals only while the separate UL signals are not faded, e.g., via a link antenna (not shown in Figure 6.3.1-1). Each output channel of the channel emulator typically requires amplification to provide enough signal level to propagate over the air to the DUT.

A turntable is normally used to provide azimuthal averaging of the device orientation within the test volume, although rotation of the channel model can also be used to accomplish a similar result, depending on the desired angular resolution. Rotation of the device within the test volume provides for determining a statistical average performance of the device in the specified spatial channel, as opposed to measuring a radiation pattern of the DUT. With dynamic channel models, time-variant angles of multipaths are rotated around the DUT, controlled by the channel emulator.

Figure 6.3.1-1: Block Diagram of a Typical Boundary Array RF Environment Simulation System

6.3.2	Minimum range length and test zone size

The minimum range length is 1.2m. While for MPAC systems, the far-field requirements do not have to apply, it was shown that the spatial correlation can be impacted significantly for distances below 1.2m.

The minimum test zone size for the NR MIMO OTA test method with dynamic link adaptation is 30cm. The DUT shall be completely contained within the test zone.

6.3.3	Ripple test for quiet zone

For an azimuthal boundary array, the standard phi-axis ripple test described in Clause D.1 of [5] shall be used with a cylindrical quiet zone 300 mm in diameter around the phi axis and 300 mm tall.

6.3.4	Calibration procedure

Clause 6.2.1.1 of [5] applies in terms of system calibration.

6.3.5	Test procedure

The communication tester shall record the TP with the maximum downlink power set to -80 dBm/15 kHz in the required 200 ms increments throughout the entire duration of the model instead of the TP being queried periodically from the control PC. The TMT trace shall be recorded as a function of time, synchronized with the start of the channel model to reliably calculate the desired CDF and subsequently the CTMT metrics, as outlined in Clause 5. After the completion of the channel model, the recorded TP results shall be made available from the communication tester to the control PC, e.g., as comma separated value list over SCPI or as file via remote file system.

6.4	UE positioning in MPAC

In order to minimize measurement uncertainty, it is important that test house ensures the DUT is oriented within the chamber’s test zone in a standardized manner. The “black-box” testing approach is adopted for NR MIMO OTA testing, i.e., the physical centre of the DUT shall be placed in the centre of the test zone.

Two aspects are considered to determine the DUT antenna spacing and positioning guidelines. The maximum antenna spacing in the DUT shall be within the limit determined by the MPAC system’s ability to emulate the spatial correlation function and the power stability of the field incident on the DUT. The DUT shall wholly reside within the 300 mm test zone under all DUT positioning conditions. Clause B.2 provides a preliminary set of normative DUT orientation conditions.

The maximum antenna spacing in the DUT shall be defined as 1.7 wavelength at the centre frequency of the band under test. Testing is not required in any band where the maximum antenna spacing of any two antennas is greater than 1.7 wavelengths.

7	Channel models

7.1	Dynamic channel model definition

The dynamic channel models, UMidyn and UMadyn, in this document assume the UE noise limit condition, i.e., UE throughput is characterized as a function of signal power, and take the full UMa/UMi path loss model of [7] into account.

Dynamic, i.e., non-stationary, channel models shall be used for evaluation of the CTMT MIMO TP Metric defined in Clause 5. The modelling principle is described in [6]. A DUT route is defined by a number of way points. Each way point is assigned a different 3GPP CDL model, together with orientations, speed and direction of travel (DoT) of the DUT. Parameter sets for each way point are given in tables of Annex A. For smooth/continuous channel modeling, each parameter, such as DUT speed, orientation, path delays, powers, Ricean K-factor, etc., must be linearly interpolated between two successive way points. Angles and angle spreads are interpolated in degree units, delays in nanoseconds, cluster powers and K-factors in linear units, and path losses and XPRs in decibel units. The LOS direction is an exception, being determined by linearly interpolated DUT and BS coordinates. However, ZoA is always 90.

Parameters are interpolated between way points 𝑎 and 𝑏 as

| εt=εaTab−t+εbtTab, | (1) |
| --- | --- |

where 𝜀(𝑡), 𝜀(𝑎), and 𝜀(𝑏) are the parameter values in the time instant 𝑡, way point 𝑎, and 𝑏, respectively, and 𝑇_𝑎𝑏 is the total time from way point 𝑎 to 𝑏 in seconds, such that 𝑡=0 at way point 𝑎 and 𝑡=𝑇_𝑎𝑏 at way point 𝑏.

To avoid discontinuities, the DoT does not follow the route geometry, but instead it is continuously interpolated according to eq. (1) where the DoT varies from the arrival direction of previous way-point to the arrival direction of the next way-point. Hence, the DoT is modelled to continuously change along a straight route segment. The DoT and UE orientation equate, i.e., the UE is oriented according to its interpolated DoT at each time instant.

Direction of the LOS path is quantized to the closest probe direction and the path will be radiated through one probe at a time. The instantaneous probe used for radiating the LOS path (when present) is determined by the interpolated LOS AoA and DoT as

| kt=argmink φk−AoALOSt−DoTt, | (2) |
| --- | --- |

where φk,  k=1,…,K, is the azimuth angle of the kth probe.

Different way point may have different number of clusters. Assume way point a has N and b has N+1 clusters. For cluster power interpolation the power of non-existing cluster N+1 in way point a is set PN+1 (a)=−100 dB and the power ramp is interpolated between a and b as defined by the equation. In LOS condition the power of LOS ray is determined by the Ricean K-factor. Transitions between LOS and NLOS conditions are handled by defining the power of LOS ray to −100 dB for NLOS way points. These 100 dB power values correspond to the cluster parameters in tables of Annex A, i.e., 100 dB is the power before adding the distance dependent path loss. For other parameter the following rule for void clusters is applied. Parameter ε(w,n) denotes the parameter value of cluster n at way point w. If cluster (w−1,n) is void and (w,n) is not, then  ε(w−1,n)=ε(w,n), i.e., the value of next way point is copied to the previous. The same applies if (w+1,n) is void and (w,n) is not, then ε(w+1,n)=ε(w,n). If both the previous and the next way point are void, then the closest non-void value is selected. Here closest means the closest in way point indices, not closest in time or distance. Notice that these clusters being void in both the previous and next way point have very low gain and will have only minimal contribution to the channel.

Unwanted randomness of channel models is removed by modifications defined in Clause 7.2 of [5]. It is, by specifying fixed 2×2 initial phases for each ray, as defined in Table 7.2-8 of [5] and introducing a uniformly distributed random scalar initial phase per path instead. Moreover, the coupling of ray angles is fixed as specified in Table 7.2-6 of [5]. Finally, after having interpolated propagation parameters, fixed randomness, and base station antenna configuration as defined in Clause 7.2 the fading channel coefficients are generated using the normal procedure defined in [7].

Overview of dynamic scenario geometry, K-factor and speed profile for UMa scenario is presented in Figures 7.1-1 and 7.1-2 and for UMi scenario in Figures 7.1-3 and 7.1-4.

Figure 7.1-1: Base station and the UE route in the UMa case. The base station location is on left and DUT route starts from the rightmost way point and continues counterclockwise back to the starting point. UE is always oriented towards its direction of travel (DoT)

Figure 7.1-2: DUT speed and maximum Doppler frequency (top). LOS AoA as observed in the DUT coordinate system (middle). Narrowband Ricean K-factor (bottom). Way points are shown by vertical dotted lines. Top figure contains the CDL model scenario label A, C, D, E on the time axis. Graphs are for the UMa scenario

Figure 7.1-3: Base station and the UE route in the UMi case. The base station location is on left and UE route starts from the rightmost way point and continues clockwise back to the starting point. UE is always oriented towards its direction of travel (DoT)

Figure 7.1-4: UE speed and maximum Doppler frequency (top). LOS AoA as observed in the UE coordinate system (middle). Narrowband Ricean K-factor (bottom). Way points are shown by vertical dotted lines. Top figure contains the CDL model scenario label A, C, D, E on the time axis. Graphs are for the UMi scenario

The cross-polarization power ratio in a propagation channel is defined as:

XPR=XPRV = XPRH

where

XPRV=SVVSHV and  XPRH=SHHSVH

and

- SVV is the coefficient for scattered/reflected power on V-polarization and incident power on V-polarization

- SVH is the coefficient for scattered/reflected power on V-polarization and incident power on H-polarization

- SHV is the coefficient for scattered/reflected power on H-polarization and incident power on V-polarization

- SHH is the coefficient for scattered/reflected power on H-polarization and incident power on H-polarization

7.2	Emulation of base station beamforming configuration

In geometry-based channel models, the antennas and propagation are specified separately. Hence, the propagation parameters specified in the annex are not dependent on the BS antenna or beam definitions. The single BS antenna element has 65 HPBW with 8 dBi gain and it is as specified in Table 7.3-1 of [7]. Moreover, no specific beamforming is included in the antenna radiation pattern. The beamforming for the PDSCH transmission is realized by the PMI-feedback based MIMO precoding by the BS emulator when the test case is configured with a dynamic link adaptation for rank and MIMO precoding. The antenna array geometry and physical antenna element mapping to logical CSI-RS antenna ports is defined and implemented in channel emulation, such that each logical CSI-RS antenna port is mapped to one channel emulation input port. The number of CSI-RS antenna ports is 4 and the CSI-RSs are non-beamformed. Therefore, only BS element radiation pattern is considered for each channel model input port. Antenna array elements are dual-polarized following polarization model 2 from [7] and the element spacing is 0.5 wavelengths. The array configuration for 4 CSI-RS antenna ports is shown in Figure 7.2-1.

Figure 7.2-1: Base station antenna array configuration

8	Channel model validation

8.1	General

<Editor’s note: Additional frequencies for validation measurements are not precluded.>

This clause describes the MIMO OTA validation measurements required to ensure that the test conditions are correctly implemented and hence capable of generating the propagation environment, as described by the model, within the designated test zone.

The channel model validations defined in this Clause apply only to the channel models defined in this document, i.e., the channel models with full path loss model [7] and the UE noise limited environment.

The following validation measurements shall be performed at 2450 MHz:

- Path Loss (PL)

- Power Delay Profile (PDP)

- Doppler/Temporal correlation (TCF)

- Spatial Correlation (SCF)

- Cross-polarization (XPO)

The target values of validation items defined in following clauses are obtained computationally from propagation parameters of Annex A using the specified antennas. The reference channel model is ideal and primarily does not consider multi-probe OTA limitations. The only exception is the LOS path, which is quantized to the closest probe of MPAC configuration as defined in Clause 7.1. This quantization affects the target statistical values of spatial correlation in Clause 8.2.5.

8.2	Validation of the dynamic channel models

8.2.1	Measurement Setup

The measurement setup includes the following equipment listed in Table 8.2.1-1.

Table 8.2.1-1: Required Measurement Equipment

| Item | Quantity | Item | PL | PDP | TCF | SCF | XPO |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Channel Emulator (CE) | x | x | x | x | x |
| 2 | 1 | Signal Generator (SG) (Optional, Note 1-2) | x | (x) | x | (x) | x |
| 3 | 1 | Signal Analyser (SAN) | x | (x) | x | (x) | x |
| 4 | 1 | Vector Network Analyser (VNA) (Note 3) | - | (x) | - | (x) | - |
| 5 | 1 | Dipole | x | x | x | x | x |
| 6 | 1 | Loop | - | - | - | - | x |
| Note 1. VNA can typically generate CW. Note 2. Some gNB emulators can generate NR/AWGN signals in non-signaling mode. Note 3. Frequency domain method uses VNA and time domain method SG + SAN. |  |  |  |  |  |  |  |

8.2.1.1	Network Analyser (VNA) Setup

Figure 8.2.1.1-1 shows a typical network analyser setup for channel model validation purposes.

Figure 8.2.1.1-1: Setup for Anechoic Chamber VNA Measurements

8.2.1.2	Signal Analyser (SAN) Setup

Figure 8.2.1.2-1 shows a typical signal generator and signal analyser setup for channel model validation purposes. Depending on CE implementation, the trigger direction between SAN and CE needs to be adjusted, i.e., from SAN to CE or from CE to SAN.

Figure 8.2.1.2-1: Setup for Signal Analyser Measurements

8.2.2	Validation of dynamic path loss

8.2.2.1	PL Method of Measurement

The intention of this validation measurement is to capture the emulated dynamic path loss (PL) as it is realized in the centre of the test zone. Measurement antenna is a vertical dipole.

The measurement antenna is placed in the centre of the test zone. Time domain technique depicted in Figure 8.2.1.2-1 is used. Table 8.2.2.1-1 outlines the settings for the channel emulator, signal generator, and signal analyser respectively.

The time domain technique (time sweep) is used for the validation. A signal generator transmits a CW signal through the test system. The CW signal is split to two input ports of fading emulator that correspond to the two first signal streams of the gNB emulator, i.e., two orthogonally polarized co-located gNB antennas. The signal is received by a test antenna within the test area. Finally, the signal is collected by a signal analyser and the measured signal is stored. Signal generator and signal analyser settings are listed in Tables 8.2.2.1-2 and 8.2.2.1-3, respectively. The measurement is triggered to start with the time instant 0 of the channel model and to stop at the last time instant of the channel model.

Table 8.2.2.1-1: MPAC Dynamic Channel Model Specification

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | 2450 |
| Channel emulator mode | N/A | Triggered start and stop |
| Channel model |  | As specified in Clause 7.1 |
| Mobile speed | km/h | Dynamic, as specified in Clause 7.1 |

Table 8.2.2.1-2: MPAC Path Loss Signal Generator Settings

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | 2450 |
| Output Power | dBm | Function of the CE. Sufficiently above Noise Floor |

Table 8.2.2.1-3: MPAC Path Loss Signal Analyser Settings

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | 2450 |
| Sampling | Hz | At least 15 times bigger than the max Doppler spread (fd=v/λ) |
| Observation time | s | One full duration of the channel model route. |

The channel emulator should issue a trigger signal when the channel model is started. When the channel model is run to the end, another trigger signal is issued. A time domain trace with the SAN is collected. Data recording is synchronized with the channel emulator trigger.

The PL validation is performed with the maximum downlink power at -80 dBm/15kHz while ensuring that the signal level exceeds the noise level during the entire dynamic PL validation.

8.2.2.2	PL Measurement Antenna

The measurement antenna shall be a vertically-oriented dipole.

8.2.2.3	PL Measurement Results Analysis

The measured channel gains (inverse of path loss) are averaged over time segments and normalized such that the maximum value across all segments is 0 dB. The resulting normalized channel gains are the target values.

Lengths of time segment and time increment between segments in seconds are specified separately for both models. Example values for start and end time of UMa route segments are given in Table 8.2.2.3-1 and of the UMi route segments are given in Table 8.2.2.3-2.

Table 8.2.2.3-1: Start and end times of time segments for UMa route

| Segment number | Start time [s] | End time [s] |
| --- | --- | --- |
| 1 | 1 | 5 |
| 2 | 10 | 15 |
| 3 | 21 | 42 |
| 4 | 47 | 52 |
| 5 | 62 | 67 |
| 6 | 82 | 87 |
| 7 | 90 | 95 |
| 8 | 102 | 107 |
| 9 | 115 | 120 |

Table 8.2.2.3-2: Start and end times of time segments for UMi route

| Segment number | Start time [s] | End time [s] |
| --- | --- | --- |
| 1 | 1 | 5 |
| 2 | 10 | 20 |
| 3 | 30 | 40 |
| 4 | 44 | 54 |
| 5 | 60 | 70 |
| 6 | 77 | 82 |
| 7 | 90 | 95 |

8.2.2.4	PL Target Values

The target values for the UMa route are specified in Table 8.2.2.4-1 and illustrated in Figure 8.2.2.4-1 while the target values for the UMi route are specified in Table 8.2.2.4-2 and illustrated in Figure 8.2.2.4-2.

Table 8.2.2.4-1: UMadyn Dynamic Path Gain (Path Loss) Target Values for the Measured Normalized Channel Gain for the UMa Route with full path loss model [7]

| Segment # | UMadyn PL Target [dB] |
| --- | --- |
| 1 | -0.8 |
| 2 | 0 |
| 3 | -29.5 |
| 4 | -27.6 |
| 5 | -21.5 |
| 6 | -30.1 |
| 7 | -0.1 |
| 8 | -19.8 |
| 9 | -0.7 |

Figure 8.2.2.4-1: UMadyn Dynamic Path Gain (Path Loss) Targets for the Measured Normalized Channel Gain for the UMa Route with full path loss model [7]

Table 8.2.2.4-2: UMidyn Dynamic Path Gain (Path Loss) Target Values for the Measured Normalized Channel Gain for the UMi Route with full path loss model [7]

| Segment # | UMidyn PL Target [dB] |
| --- | --- |
| 1 | -7.5 |
| 2 | 0 |
| 3 | -16.6 |
| 4 | -12.9 |
| 5 | -16.5 |
| 6 | -0.9 |
| 7 | -0.7 |

Figure 8.2.2.4-2: UMidyn Dynamic Path Gain (Path Loss) Targets for the Measured Normalized Channel Gain for the UMi Route with full path loss model [7]

8.2.3	Validation of Power Delay Profile (PDP)

8.2.3.1	General

The purpose of this item is to validate the slow variation of power delay profile as observed by the DUT due to the dynamic channel model. PDP can be measured either by using the frequency sweep method and a network analyser or alternatively the time domain method with wideband signal transmitted through the test system. In both options the test signal is received by a test antenna in the test zone. The variation of PDP in the reference channel model is caused by the change of path delays and powers along the route. There are two categories of channel segments (defined in Tables 8.2.2.3-1 and 8.2.2.3-2) in the UMadyn and UMidyn models. Firstly, those with same CDL model throughout the segment, meaning that only the initial delay changes during the segment but excess delays remain unchanged, i.e., quasi time invariant. Secondly, some segments have different CDL models on preceding and subsequent way-points, meaning that both the initial delay and excess delays are time variant during the segment. Channel segments with quasi time invariant and time variant excess delays are validated differently.

In the UMadyn model the channel segments 1, 2, 6, 7, and 9 are in the first category and segments 3, 4, 5, and 8 in the second. In the UMidyn model, the channel segments 2, 6, and 7 are in the first category and segments 1, 3, 4, and 5 in the second.

The validation is done by observing the measured channel impulse responses and averaging them to a PDP estimate. PDP can be evaluated either by using the frequency sweep method and a network analyser or alternatively the time domain method with wideband signal transmitted through the test system. In both options the test signal is received by a test antenna in the test zone.

8.2.3.2	PDP Method of Measurement using Frequency-Domain Technique

A network analyser transmits frequency tones through the test system when the frequency sweep technique is used in the validation. See Figure 8.2.1.1-1 for the block diagram. The signal is received by a test antenna within the test area. During each frequency sweep the channel model is paused by the channel emulator. After the sweep, channel model is stepped to the next time instant and paused again for the next frequency sweep. The time increment between consecutive frequency sweeps measuring the frequency responses of channel model instances is defined in Table 8.2.3.2-1. It is at most the inverse of twice the maximum Doppler frequency, i.e., Δt≤1(2fmax).

Channel model segments are specified in Tables 8.2.2.3-1 and 8.2.2.3-2. At the first time instant of a segment the model is paused and the complex channel frequency response is measured and stored. This is repeated with the specified time increment until the whole time segment is covered. The sequence of measured channel responses of the segment is analysed as defined in Clause 8.2.3.5. The subsequent channel segments are measured then correspondingly. Network analyser settings are defined in Table 8.2.3.2-1. The measured channel frequency response of channel model segment s at frequency f and channel model time instant t is

Hsf,t=Hsf0+nΔf,ts,0+m Δt

where f0 is the lowest frequency point, n=0,1,…,N−1 is the index of frequency point, Δf is the spacing between frequency points, ts,0 is the first time instant of segment s, m=0,1,…,Ms−1 is the index of time instants, and Δt is the increment between measured time instants.

Table 8.2.3.2-1: MPAC PDP Network Analyser Settings

| Item | Unit | Value |
| --- | --- | --- |
| Bandwidth | MHz | 200 |
| Number of frequency points | - | ≥1101 |
| Number of traces | - | One per every CIR within the segment |

8.2.3.3	PDP Method of Measurement using Time-Domain Technique

<Editor’s note: The detailed procedure will be updated at a later time.>

This clause is to define the PDP Method of Measurement using Time-Domain Technique.

8.2.3.4	PDP Measurement Antenna

The measurement antenna shall be a vertically-oriented dipole.

8.2.3.5	PDP Measurement Results Analysis using Frequency-Domain Technique

The first operation is to remove the quasi time-variant initial delay from measured responses Hsf,t. The initial delay τit at each time instant t is calculated by dividing the instantaneous link distance with the speed of light (in vacuum). It is removed from each frequency response by time shifting in frequency domain and a new delay shifted response is

H'sf,t=Hsf,texpj2πfτit.

These responses are then transformed to impulse responses

ℎsτ,t=ℎsuΔτ,t

By the inverse Fourier transformation, where Δτ= 5 ns is the width of a delay bin and u=0,1,2,… is the index of the delay bin. Note that due to different cable lengths, calibration etc. of test setups the resulting measured PDPs must be further shifted such that the first path component of PDPs is set on zero excess delay. This can be done either by visual observation or by correlating the resulting dynamic PDPs with theoretical reference PDPs and choosing the delay shift yielding best correlation.

Segments with quasi time invariant excess delays for UMa route: 1, 2, 6, 7, and 9 and for UMi route: 2, 6, and 7:

Power delay profile for each segment is determined by the time average

P'sτ=1Msm=0Ms−1ℎsτ,ts,0+m Δt2.

Finally, the normalized PDP is

Psτ=P'sτmaxP'sτ.

All values of Psτ below −40 dB are neglected in validation. Peaks, i.e., local maxima of PsτdB (in dB scale) are identified and chosen as the target value of the time segment.

Segments with time variant excess delays for UMa are 3, 4, 5, and 8 and for UMi are 1, 3, 4, and 5:

These validation segment must be treated differently since straightforward averaging of the power per delay bin over the time domain would provide erroneous result. Therefore, PDPs are evaluated per cluster (tap) index not per delay bin in the case of time variant excess delays. The procedure is as follows. First, the excess delay at the first τlts,0 and the last τlts,0+ΔtMs−1 measured time instant of the segment is determined from time interpolated model parameters for each cluster l. The excess delay of lth cluster is linearly interpolated for each time sample of the measurement and then quantized to the delay resolution Δτ supported by the measurement. The resulting quantized cluster excess delay at mth time sample is τ'l,m. Then dynamic powers of l=1,…, Ls clusters on segment s are calculated as the sum of powers over time samples in delay bins determined by τ'l,m. Since cluster powers are dispersed across neighbouring delay bins due to limited measurement bandwidth, also the previous and the successive delay bin are included in the summation. This is expressed by the formula

P's,l=m=0Ms−1ℎsτ'l,m−Δτ ,ts,0+m Δt2+ℎsτ'l,m ,ts,0+m Δt2+ℎsτ'l,m+Δτ,ts,0+m Δt2.

Finally, the measured power per cluster is normalized such that the maximum cluster power per segment becomes unity

Ps,l=P's,lmaxP's,1, …,P's,Ls .

8.2.3.6	PDP Measurement Results Analysis using Time-Domain Technique

<Editor’s note: The detailed procedure will be updated at a later time.>

This clause is to define the PDP Measurement Results Analysis using Time-Domain Technique.

8.2.3.7	PDP Target Values

The target values for the UMa route are specified in Table 8.2.3.7-1 and illustrated in Figure 8.2.3.7-1 which uses the time segments defined in Table 8.2.2.3-1; the original PDP values without any processing related to the delay resolution are tabulated in Table 8.2.3.7-2. The target values for the UMi route are specified in Table 8.2.3.7-3 and illustrated in Figure 8.2.3.7-2 which uses the time segments defined in Table 8.2.2.3-2; the original PDP values without any processing related to the delay resolution are tabulated in Table 8.2.3.7-4. Note that red lines in the figures denote the power per excess delay and blue lines the power per cluster index. Entries of the table contain either discrete excess delay/power pairs or cluster index/power pairs, depending on the segment type. In the case of segments with time variant excess delays the initial and final excess delays of clusters must be as specified in the model parameter tables in Annex A and rounded to the delay resolution supported by the measurement bandwidth (1/200 MHz = 5 ns). The window used in the bandwidth filtering is a Chebyshev window with 120 dB sidelobe factor, discussed in some more detail in Clause 8.2.3.8.

Figure 8.2.3.7-1 Target PDP Segments of the UMadyn Channel Model with full path loss model [7]

Table 8.2.3.7-1 UMadyn PDP Targets of the UMa Route with full path loss model [7]

| Segment # |  | UMadyn Target PDP |
| --- | --- | --- |
| 1 | Delay τ [ns] | 0, 130, 170, 245, 380, 745, 885 |
|  | Power P1τ [dB] | 0, -18.8, -19.8, -22.6, -36.0, -25.8, -32.1 |
| 2 | Delay τ [ns] | 0, 130, 170, 245, 380, 745, 885 |
|  | Power P2τ [dB] | 0, -19.4, -20.6, -23.2, -32.8, -24.1, -34.7 |
| 3 | Cluster index l | 1, 2, ..., 23 |
|  | Power P3,l [dB] | -9.8, 0.0, -1.9, -6.3, -8.2, -6.6, -3.3, -8.9, -8.3, -11.7, -8.5, -16.3, -10.0, -13.4, -12.1, -12.8, -29.3, -29.0, -18.4, -19.3, -17.5, -31.9, -29.8 |
| 4 | Cluster index l | 1, 2, ..., 23 |
|  | Power P4,l [dB] | -15.0, 0.0, -2.0, -9.7, -11.8, -14.2, -3.9, -13.3, -6.9, -20.8, -6.5, -16.0, -19.0, -17.5, -12.3, -11.3, -28.2, -29.1, -22.3, -18.2, -17.2, -30.3, -28.0 |
| 5 | Cluster index l | 1, 2, ..., 23 |
|  | Power P5,l [dB] | -8.2, -0.8, -3.7, -3.5, -5.2, 0.0, -1.6, -3.7, -10.4, -10.1, -13.4, -22.5, -7.9, -13.5, -16.2, -18.9, -23.1, -21.4, -23.4, -22.5, -20.3, -26.5, -33.1 |
| 6 | Delay τ [ns] | 0, 80, 235, 300, 340, 445, 475, 790, 985, 1550, 1675, 2040, 2295, 2415, 2565] |
|  | Power P6τ [dB] | -14.4, -3.9, 0.0, -14.0, -34.7, -17.5, -21.2, -26.5, -19.6, -19.3, -17.8, -22.1, -20.5, -22.4, -30.8 |
| 7 | Delay τ [ns] | 0, 50, 175, 240, 335, 495, 1090, 1875 |
|  | Power P7τ [dB] | 0, -16.2, -14.8, -21.0, -25.2, -19.9, -32.9, -32.9 |
| 8 | Cluster index l | 1, 2, ..., 23 |
|  | Power P8,l [dB] | 0.0, -5.7, -7.9, -11.5, -15.2, -15.4, -9.1, -14.1, -13.9, -19.6, -12.2, -19.9, -25.2, -20.9, -26.6, -27.9, -23.1, -25.6, -26.3, -33.3, -35.9, -27.5, -38.8 |
| 9 | Delay τ [ns] | 0, 60, 130, 170, 245, 380, 745, 885, 1180 |
|  | Power P9τ [dB] | 0, -36.3, -18.2, -19.0, -22.0, -38.8, -27.2, -29.8, -38.9 |

Table 8.2.3.7-2 UMadyn Ideal PDP of the UMa Route with full path loss model [7]

| Segment # |  | UMadyn Ideal PDP |
| --- | --- | --- |
| 1 | Delay τ [ns] | 0, 0, 3, 58, 132, 167, 170, 244, 380, 747, 887 |
|  | Power P1τ [dB] | 0, -13.38, -37.64, -39.84, -18.35, -25.97, -20.55, -22.35, -35.81, -25.32, -31.66 |
| 2 | Delay τ [ns] | 0, 132, 167, 170, 244, 380, 747, 887 |
|  | Power P2τ [dB] | 0, -13.37, -18.99, -27.69, -21.19, -22.99, -32.56, -23.66, -34.28 |
| 3 | Cluster index l | 1, 2, ..., 23 |
|  | Power P3,l [dB] | -9.8, 0.0, -1.9, -6.3, -8.2, -6.6, -3.3, -8.9, -8.3, -11.7, -8.5, -16.3, -10.0, -13.4, -12.1, -12.8, -29.3, -29.0, -18.4, -19.3, -17.5, -31.9, -29.8 |
| 4 | Cluster index l | 1, 2, ..., 23 |
|  | Power P4,l [dB] | -15.0, 0.0, -2.0, -9.7, -11.8, -14.2, -3.9, -13.3, -6.9, -20.8, -6.5, -16.0, -19.0, -17.5, -12.3, -11.3, -28.2, -29.1, -22.3, -18.2, -17.2, -30.3, -28.0 |
| 5 | Cluster index l | 1, 2, ..., 23 |
|  | Power P5,l [dB] | -8.2, -0.8, -3.7, -3.5, -5.2, 0.0, -1.6, -3.7, -10.4, -10.1, -13.4, -22.5, -7.9, -13.5, -16.2, -18.9, -23.1, -21.4, -23.4, -22.5, -20.3, -26.5, -33.1 |
| 6 | Delay τ [ns] | 0, 76, 79, 81, 85, 232, 235, 239, 240, 289, 299, 340, 447, 476, 790, 987, 1551, 1675, 1999, 2042, 2296, 2417, 2564 |
|  | Power P6τ [dB] | -11.50, -4.04, -8.52, -6.34, -8.04, 0.00, -2.20, -3.90, -7.18, -18.30, -11.17, -31.84, -14.35, -18.25, -23.64, -16.48, -16.34, -14.89, -38.14, -18.98, -17.56, -19.24, -27.89] |
| 7 | Delay τ [ns] | 0, 0, 47, 49, 49, 51, 65, 173, 175, 178, 240, 337, 495, 1091, 1876 |
|  | Power P7τ [dB] | 0, -21.78, -19.32, -21.62, -26.21, -23.32, -21.13, -17.54, -19.74, -21.54, -21.01, -24.89, -19.88, -32.82, -32.75 |
| 8 | Cluster index l | 1, 2, ..., 23 |
|  | Power P8,l [dB] | 0.0, -5.7, -7.9, -11.5, -15.2, -15.4, -9.1, -14.1, -13.9, -19.6, -12.2, -19.9, -25.2, -20.9, -26.6, -27.9, -23.1, -25.6, -26.3, -33.3, -35.9, -27.5, -38.8 |
| 9 | Delay τ [ns] | 0, 0, 3, 58, 128, 132, 167, 170, 244, 380, 747, 887, 1178 |
|  | Power P9τ [dB] | 0, -13.38, -33.69, -35.89, -37.69, -17.78, -24.44, -19.98, -21.78, -38.64, -26.76, -29.33, -38.52 |

Figure 8.2.3.7-2 Target PDP Segments of the UMidyn Channel Model with full path loss model [7]

Table 8.2.3.7-3 UMidyn PDP Targets of the UMi Route with full path loss model [7]

| Segment # |  | UMidyn Target PDP |
| --- | --- | --- |
| 1 | Cluster index l | 1, 2, ...17 |
|  | Power P1,l [dB] | 0, -21.8, -22.5, -24.3, -16.8, -16.7, -16.5, -18.3, -25.5, -24.5, -29.6, -24, -30, -34.7, -39.9, -39.8, -37.7 |
| 2 | Delay τ [ns] | 0, 35, 75, 95, 140, 430, 510, 680 |
|  | Power P2τ [dB] | 0, -35.3, -17.5, -18.5, -21.5, -28.9, -28.2, -37.5 |
| 3 | Cluster index l | 1, 2, ..., 23 |
|  | Power P3,l [dB] | -17.1, 0, -2.8, -2.8, -4.8, -1.6, -2, -4.4, -5.9, -9, -6.1, -26.4, -7.5, -21.5, -20.3, -10.7, -15.2, -16.2, -16.8, -16.8, -15.5, -19.2, -23.6 |
| 4 | Cluster index l | 1, 2, ..., 23 |
|  | Power P4,l [dB] | -11.3, 0, -2.5, -4.3, -6.1, -2, -2, -5.2, -7.8, -11.5, -8.4, -21.8, -9.3, -15.9, -16.6, -13.6, -21, -20.8, -19.5, -19.3, -18, -24.4, -27.7 |
| 5 | Cluster index l | 1, 2, ..., 23 |
|  | Power P5,l [dB] | -11, 0, -2, -5.6, -7.5, -5.9, -3.1, -8.2, -8.1, -11.2, -8.4, -17.6, -9.4, -14.4, -13.3, -12.5, -27.1, -26.7, -17.5, -18.8, -16.6, -30.1, -28.9 |
| 6 | Delay τ [ns] | 0, 30, 100, 140, 195, 285 |
|  | Power P6τ [dB] | 0, -20.4, -17, -26.1, -33.6, -20.1 |
| 7 | Delay τ [ns] | 0, 30, 100, 140, 195, 285 |
|  | Power P7τ [dB] | 0, -20.7, -16.2, -24.4, -30.6, -20.1 |

Table 8.2.3.7-4 UMidyn Ideal PDP of the UMi Route with full path loss model [7]

| Segment # |  | UMidyn Ideal PDP |
| --- | --- | --- |
| 1 | Cluster index l | 1, 2, ...17 |
|  | Power P1,l [dB] | 0, -21.8, -22.5, -24.3, -16.8, -16.7, -16.5, -18.3, -25.5, -24.5, -29.6, -24, -30, -34.7, -39.9, -39.8, -37.7 |
| 2 | Delay τ [ns] | 0, 0, 2, 33, 74, 76, 96, 98, 141, 431, 511, 679] |
|  | Power P2τ [dB] | 0, -13.37, -32.64, -34.84, -36.64, -17.26, -23.33, -19.46, -21.26, -28.65, -27.97, -37.28 |
| 3 | Cluster index l | 1, 2, ..., 23 |
|  | Power P3,l [dB] | -17.1, 0, -2.8, -2.8, -4.8, -1.6, -2, -4.4, -5.9, -9, -6.1, -26.4, -7.5, -21.5, -20.3, -10.7, -15.2, -16.2, -16.8, -16.8, -15.5, -19.2, -23.6 |
| 4 | Cluster index l | 1, 2, ..., 23 |
|  | Power P4,l [dB] | -11.3, 0, -2.5, -4.3, -6.1, -2, -2, -5.2, -7.8, -11.5, -8.4, -21.8, -9.3, -15.9, -16.6, -13.6, -21, -20.8, -19.5, -19.3, -18, -24.4, -27.7 |
| 5 | Cluster index l | 1, 2, ..., 23 |
|  | Power P5,l [dB] | -11, 0, -2, -5.6, -7.5, -5.9, -3.1, -8.2, -8.1, -11.2, -8.4, -17.6, -9.4, -14.4, -13.3, -12.5, -27.1, -26.7, -17.5, -18.8, -16.6, -30.1, -28.9 |
| 6 | Delay τ [ns] | 0, 0, 27, 29, 29, 30, 37, 100, 101, 103, 138, 195, 286 |
|  | Power P6τ [dB] | 0, -21.78, -35.18, -37.48, -21.47, -39.18, -25.24, -19.88, -22.08, -23.88, -25.87, -33.54, -20.01 |
| 7 | Delay τ [ns] | 0, 0, 27, 29, 29, 30, 37, 100, 101, 103, 138, 195, 286 |
|  | Power P7τ [dB] | 0, -21.82, -29.86, -32.16, -23.40, -33.86, -23.77, -19.06, -21.26, -23.06, -24.14, -30.53, -20.01 |

8.2.3.8	Chebyshev Window Function

The Chebyshev window is a finite impulse response (FIR) window for L samples designed to minimize the main-lobe width for a specified side-lobe level, r, in decibels. It is based on the Chebyshev polynomials of the first kind. The Chebyshev window is designed such that all side-lobes in its frequency response have equal height (equiripple behaviour), and its main lobe is as narrow as possible for a given side-lobe attenuation level. The window coefficients are symmetric around the centre.

The Chebyshev window is defined in the frequency domain as:

W(ω)=coshMcosh−1βcosω2coshMcosh−1β

where

M=L−1

and

β=cosh1Mcosh−110r20

A prominent function to create this window is the chebwin function in MATLAB®, e.g, w=chebwin(2001,120) which creates a 2001-point Chebyshev window with 120 dB of side-lobe attenuation as shown in Figure 8.2.3.8-1.

Figure 8.2.3.8-1: Visualization of the Chebyshev window with 120dB of side-lobe attenuation using MATLAB®

8.2.4	Validation of Doppler/Temporal Correlation

8.2.4.1	TCF Method of Measurement

The purpose of this item is to validate the slow variation of Doppler effect due of the dynamic channel model. This is done indirectly by observing the temporal correlation function (TCF). TCF at a few time lags are evaluated from the measured narrowband signal transmitted through the test system.

The time domain technique (time sweep) is used for the validation. See the block diagram of the setup in Figure 8.2.1.2-1. A signal generator transmits a CW signal through the test system. The signal is received by a test antenna within the test area. Finally, the signal is collected by a signal analyser and the measured signal is stored. Signal analyser and signal generator settings are defined in Tables 8.2.4.1-1 and 8.2.4.1-2. The measurement is triggered to start with the time instant 0 of the channel model and to stop at the last time instant of the channel model.

Table 8.2.4.1-1: MPAC TCF Signal Generator Settings

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | 2450 |
| Output Power | dBm | Function of the CE. Sufficiently above Noise Floor |

Table 8.2.4.1-2: MPAC TCF Signal Analyser Settings

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | 2450 |
| Sampling | Hz | At least 15 times bigger than the max Doppler spread (fd=v/λ) |
| Observation time | s | One full duration of the channel model route. |

The signal analyser records I/Q samples at the rate of 15 times the maximum Doppler frequency (72.24 Hz). The full model length is measured at once.

8.2.4.2	TCF Measurement Antenna

The measurement antenna shall be a vertically-oriented dipole.

8.2.4.3	TCF Measurement Results Analysis

Segments of recorded I/Q samples are selected, autocorrelation of I/Q samples of a segment is calculated, and a few samples of the resulting temporal correlation function are picked for the validation. Time lags to be picked for TFC Δt are 4.6 ms and 20.2 ms.

8.2.4.4	TCF Target Values

The target values for the UMa route are specified in Table 8.2.4.4-1 and illustrated in Figure 8.2.4.4-1 which uses the time segments defined in Table 8.2.2.3-1. The target values for the UMi route are specified in Table 8.2.4.4-2 and illustrated in Figure 8.2.4.4-2 which uses the time segments defined in Table 8.2.2.3-2. Estimated temporal correlation values at time lag Δt=4.6 ms and Δt=20.2 ms are illustrated in the top and bottom figure, respectively. Target values are shown within time segment limits.

Figure 8.2.4.4-1: UMadyn Temporal Correlation Function of UMa Route for Two Different TFC Time Lags

Table 8.2.4.4-1: Dynamic UMadyn Temporal Correlation Targets of the UMa Route with full path loss model [7]

| Segment # | UMadyn Target TCF at Δt=4.6 ms | UMadyn Target TCF at Δt=20.2 ms |
| --- | --- | --- |
| 1 | 0.970 | 0.876 |
| 2 | 0.961 | 0.879 |
| 3 | 0.994 | 0.886 |
| 4 | 0.436 | 0.355 |
| 5 | 0.648 | 0.345 |
| 6 | 0.521 | 0.100 |
| 7 | 0.801 | 0.228 |
| 8 | 0.837 | 0.398 |
| 9 | 0.939 | 0.616 |

Figure 8.2.4.4-2: UMidyn Temporal Correlation Function of UMi Route for Two Different TFC Time Lags

Table 8.2.4.4-2: Dynamic UMidyn Temporal Correlation Targets of the UMi Route with full path loss model [7]

| Segment # | Target UMidyn TCF at Δt=4.6 ms | Target UMidyn TCF at Δt=20.2 ms |
| --- | --- | --- |
| 1 | 0.871 | 0.436 |
| 2 | 0.952 | 0.857 |
| 3 | 0.513 | 0.188 |
| 4 | 0.698 | 0.064 |
| 5 | 0.744 | 0.374 |
| 6 | 0.959 | 0.813 |
| 7 | 0.965 | 0.854 |

8.2.5	Validation of Spatial Correlation

8.2.5.1	SCF Method of Measurement

The purpose of this item is to validate the slow variation of angular power distribution as observed by the DUT due of the dynamic channel model. The variation of PAS in the reference channel model is caused by the change of path angles, path powers, and UE orientation along the route. The validation is done indirectly by observing the spatial correlation function (SCF). SCF is evaluated from the measured narrowband signal transmitted through the test system and received by a test antenna in a few spatial positions within the test zone.

The time domain technique (time sweep) is used for the validation. See the block diagram of the setup in Figure 8.2.1.2-1. A signal generator transmits a CW signal through the test system. The CW signal is split to two input ports of fading emulator that correspond to the two first signal streams of the gNB emulator. The signal is received by a test antenna in a specific position within the test area. Finally, the signal is collected by a signal analyser and the measured signal is stored. Signal analyser and signal generator settings are defined in Tables 4.2.3.1-1 and 4.2.3.1-2. The measurement is triggered to start with the time instant 0 of the channel model and to stop at the last time instant of the channel model. The position of test antenna is changed and the measurement is repeated. All spatial positions are illustrated in Figure 8.2.5.1-1.

Table 8.2.5.1-1: MPAC SCF Signal Generator Settings

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | 2450 |
| Output Power | dBm | Function of the CE. Sufficiently above Noise Floor |

Table 8.2.5.1-2: MPAC SCF Signal Analyser Settings

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | 2450 |
| Sampling | Hz | At least 15 times bigger than the max Doppler spread (fd=v/λ) |
| Observation time | s | One full duration of the channel model route. |

The full model length is measured at once.

Figure 8.2.5.1-1: Spatial sampling points within the test zone at 2450 MHz

Table 8.2.5.1-3: Spatial sample points, i.e., positions of test antenna for the SCF validation at 2450 MHz

| Point number | x [mm] | y [mm] | z [mm] |
| --- | --- | --- | --- |
| #1 (reference point) | 0 | -150 | 0 |
| #2 | -22.88 | -148.29 | 0 |
| #3 | -86.27 | -122.71 | 0 |
| #4 | -149.90 | -5.73 | 0 |

8.2.5.2	SCF Measurement Antenna

The measurement antenna shall be a vertically-oriented dipole

8.2.5.3	SCF Measurement Results Analysis

Time segments of recorded I/Q samples are selected. For each time segment the cross correlation (with zero time lag) of I/Q samples measured in different spatial sample points is calculated. Spatial sample points picked for SFC have at maximum Euclidian distance Dd = 1.7 wavelength to the reference sample point. Absolute values of estimated complex spatial correlations per time segment are chosen as the target SCF values.

8.2.5.4	SCF Target Values

The target values for the UMa route are specified in Table 8.2.5.4-1 and illustrated in Figure 8.2.5.4-1, which uses the time segments along the dynamic UMadyn model proposed in Table 8.2.2.3-1. The target values for the UMi route are specified in Table 8.2.5.4-2 and illustrated in Figure 8.2.5.4-2, which uses the time segments along the dynamic UMidyn model proposed in Table 8.2.2.3-2. Estimated spatial correlation values at spatial spacing Δd=22.9 mm, Δd=90.5 mm, and Δd=208.0 mm (referenced to reference point #1) are illustrated in Figures 8.2.5.4-1 and 8.2.5.4-2. Target values are shown within time segment limits.

Figure 8.2.5.4-1: UMadyn Spatial Correlation Function of UMa Route with full path loss model [7] for Three Different SFC Test Antenna Positions

Table 8.2.5.4-1: UMadyn Spatial Correlation Targets of the UMa Route with full path loss model [7] for 2450 MHz

| Segment # | Target SCF at Δd=22.9 mm =0.19 λ | Target SCF at Δd=90.5 mm =0.74 λ | Target SCF at Δd=208.0 mm =1.70 λ |
| --- | --- | --- | --- |
| 1 | 0.986 | 0.908 | 0.932 |
| 2 | 0.963 | 0.611 | 0.326 |
| 3 | 0.800 | 0.159 | 0.111 |
| 4 | 0.699 | 0.721 | 0.492 |
| 5 | 0.831 | 0.414 | 0.156 |
| 6 | 0.794 | 0.392 | 0.454 |
| 7 | 0.928 | 0.482 | 0.366 |
| 8 | 0.835 | 0.285 | 0.610 |
| 9 | 0.959 | 0.575 | 0.220 |

Figure 8.2.5.4-2: UMidyn Spatial Correlation Function of UMi Route with full path loss model [7] for Three Different SFC Test Antenna Positions

Table 8.2.5.4-2: UMidyn Spatial Correlation Targets of the UMi Route with full path loss model [7] for 2450 MHz

| Segment # | Target SCF at Δd=22.9 mm =0.19 λ | Target SCF at Δd=90.5 mm =0.74 λ | Target SCF at Δd=208.0 mm =1.70 λ |
| --- | --- | --- | --- |
| 1 | 0.944 | 0.755 | 0.733 |
| 2 | 0.981 | 0.936 | 0.937 |
| 3 | 0.787 | 0.221 | 0.162 |
| 4 | 0.764 | 0.329 | 0.320 |
| 5 | 0.856 | 0.577 | 0.288 |
| 6 | 0.973 | 0.809 | 0.684 |
| 7 | 0.982 | 0.951 | 0.943 |

8.2.6	Validation of Cross-Polarization

8.2.6.1	XPO Method of Measurement

This measurement checks how well the measured vertically or horizontally polarized power levels follow expected values of cross polarization (XPO).

The time domain technique (time sweep) is used for the validation. See the block diagram of the setup in Figure 8.2.1.2-1. A signal generator transmits a CW signal through the test system. The CW signal is split to two input ports of fading emulator that correspond to the two first signal streams of the gNB emulator, i.e., that correspond to two co-located ±45° slanted linearly polarized elements. The signal is received by a vertically polarized test antenna within the test area. Finally, the signal is collected by a signal analyser and the measured signal is stored. Signal generator and signal analyser settings are listed in Tables 8.2.6.1-1 and 8.2.6.1-2. The measurement is triggered to start with the time instant 0 of the channel model and to stop at the last time instant of the channel model. The measurement is repeated with a horizontally polarized test antenna, placed in the same position. The result is a stored sequence of channel gains of segment s received with the vertically and horizontally polarized test antennas are denoted ℎV,st=ℎV,sts,0+m Δt and ℎH,st=ℎH,sts,0+m Δt, respectively, where m=0,…,Ms Note that the time increment Δt and correspondingly the number of stored time samples is different in this measurement as compared to the PDP measurement.

Table 8.2.6.1-1: MPAC path loss Signal Generator Settings

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | 2450 |
| Output Power | dBm | Function of the CE. Sufficiently above Noise Floor |

Table 8.2.6.1-2: MPAC path loss Signal Analyser Settings

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | 2450 |
| Sampling | Hz | At least 15 times bigger than the max Doppler spread (fd=v/λ) |
| Observation time | s | One full duration of the channel model route. |

8.2.6.2	XPO Measurement Antenna

Measurement antennas shall be a vertically-oriented dipole and a loop.

8.2.6.3	XPO Measurement Results Analysis

The measured channel gains (inverse of path loss) are averaged over each time segment s to obtain average powers

PV,s=1Msm=0Ms−1ℎV,sts,0+m Δt2PH,s=1Msm=0Ms−1ℎH,sts,0+m Δt2.

The resulting V/H power ratios

Xs=PV,sPH,s

of time segments s are the target values.

8.2.6.4	XPO Target Values

The target values for the UMa route are specified in Table 8.2.6.4-1 and illustrated in Figure 8.2.6.4-1 which uses the time segments defined in Table 8.2.2.3-1. The target values for the UMi route are specified in Table 8.2.6.4-2 and illustrated in Figure 8.2.6.4-2 which uses the time segments defined in Table 8.2.2.3-2.

Figure 8.2.6.4-1: Simulated Dynamic Narrowband UMadyn Polarization Power Ratios with Time Segment Limits for the UMa Route with full path loss model [7]

Table 8.2.6.4-1: UMadyn Polarization Power Ratio Target Values for the Measured Channel Gain with V and H Polarized Test Antenna for the UMa Route with full path loss model [7]

| Segment # | UMadyn XPO Target [dB] |
| --- | --- |
| 1 | 22.2 |
| 2 | 22.4 |
| 3 | 8.5 |
| 4 | 9.5 |
| 5 | 7.4 |
| 6 | 7.0 |
| 7 | 18.5 |
| 8 | 12.2 |
| 9 | 22.0 |

Figure 8.2.6.4-2: Simulated Dynamic Narrowband UMidyn Polarization Power Ratios with Time Segment Limits for the UMi Route with full path loss model [7]

Table 8.2.6.4-2: UMidyn Polarization Power Ratio Target Values for the Measured Channel Gain with V and H Polarized Test Antenna for the UMi Route with full path loss model [7]

| Segment # | UMidyn XPO Target [dB] |
| --- | --- |
| 1 | 18 |
| 2 | 21.7 |
| 3 | 8.6 |
| 4 | 8.1 |
| 5 | 8.5 |
| 6 | 21.1 |
| 7 | 20.7 |

8.3	Validation pass/fail criteria

8.3.1	General

This clause defines the pass/fail limit of FR1 MPAC system for FR1 dynamic channel model validation.

8.3.2	Pass/Fail Criteria of dynamic path loss

This clause defines the pass/fail criteria of dynamic path loss.

The dynamic path loss pass/fail limit is specified as ± 2 dB.

8.3.3	Pass/Fail Criteria of PDP

This clause defines the pass/fail criteria of PDP. The detailed pass/fail limits for each cluster of CDL-C UMadyn and CDL-C UMidyn are defined in Table 8.3.3-1.

Table 8.3.3-1: PDP pass/fail limits for CDL-C UMadyn and CDL-C UMidyn channel model validation

|  | Power Tolerance | Delay Tolerance |
| --- | --- | --- |
| Paths from 0dB to 10dB | ±1dB | ±6ns |
| Paths from 10dB to 20dB | ±2.5dB | ±6ns |
| Paths from 20dB to 30dB | ±5dB | ±6ns |
| Paths from 30dB to 40dB | ±10dB | ±6ns |

8.3.4	Pass/Fail Criteria of Doppler/Temporal correlation

This clause defines the pass/fail criteria of doppler/temporal correlation.

The pass/fail limits for theoretical temporal correlation defined in Clause 8.2.4.4 above 0.3 are formed as bands of ±0.1 of correlation capped at 1 at the high end. Additionally, when the theoretical temporal correlation drops below 0.3, the limits are formed at bands of ±0.3 of correlation capped at 0 at the low end. The pass/fail limits are tabulated in Tables 8.3.4-1 and 8.3.4-2 for the UMa and UMi routes, respectively.

Table 8.3.4-1: Dynamic UMadyn Temporal Correlation Pass/Fail Limits of the UMa Route with full path loss model [7]

| Segment # | UMadyn TCF at Δt=4.6 ms |  | UMadyn TCF at Δt=20.2 ms |  |
| --- | --- | --- | --- | --- |
|  | Lower | Upper | Lower | Upper |
| 1 | 0.870 | 1 | 0.776 | 0.976 |
| 2 | 0.861 | 1 | 0.779 | 0.979 |
| 3 | 0.894 | 1 | 0.786 | 0.986 |
| 4 | 0.336 | 0.536 | 0.255 | 0.455 |
| 5 | 0.548 | 0.748 | 0.245 | 0.445 |
| 6 | 0.421 | 0.621 | 0.000 | 0.400 |
| 7 | 0.701 | 0.901 | 0 | 0.528 |
| 8 | 0.737 | 0.937 | 0.298 | 0.498 |
| 9 | 0.839 | 1 | 0.516 | 0.716 |

Table 8.3.4-2: Dynamic UMidyn Temporal Correlation Pass/Fail Limits of the UMi Route with full path loss model [7]

| Segment # | UMidyn TCF at Δt=4.6 ms |  | UMidyn TCF at Δt=20.2 ms |  |
| --- | --- | --- | --- | --- |
|  | Lower | Upper | Lower | Upper |
| 1 | 0.771 | 0.971 | 0.336 | 0.536 |
| 2 | 0.852 | 1 | 0.757 | 0.957 |
| 3 | 0.413 | 0.613 | 0.188 | 0 |
| 4 | 0.598 | 0.798 | 0.364 | 0 |
| 5 | 0.644 | 0.844 | 0.274 | 0.474 |
| 6 | 0.859 | 1 | 0.713 | 0.913 |
| 7 | 0.865 | 1 | 0.754 | 0.954 |

8.3.5	Pass/Fail Criteria of Spatial correlation

This clause defines the pass/fail criteria of spatial correlation.

The pass/fail limits for spatial correlation are formed as bands of ±10% of correlation capped at 100% for the upper limit for target correlation defined in Clause 8.2.5.4 of 35% and above. For target correlations below 35%, the band is widened to ±20% capped at 0%. The pass/fail limits are tabulated in Tables 8.3.5-1 and 8.3.5-2 for the UMa and UMi routes, respectively.

Table 8.3.5-1: UMadyn Spatial Correlation Pass/Fail Limits of the UMa Route with full path loss model [7] for 2450 MHz

| Segment # | SCF at Δd=22.9 mm =0.19 λ |  | SCF at Δd=90.5 mm =0.74 λ |  | SCF at Δd=208.0 mm =1.70 λ |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Lower | Upper | Lower | Upper | Lower | Upper |
| 1 | 0.887 | 1 | 0.817 | 0.999 | 0.839 | 1 |
| 2 | 0.867 | 1 | 0.550 | 0.672 | 0.261 | 0.391 |
| 3 | 0.720 | 0.88 | 0.127 | 0.191 | 0.089 | 0.133 |
| 4 | 0.629 | 0.769 | 0.649 | 0.793 | 0.443 | 0.541 |
| 5 | 0.748 | 0.914 | 0.373 | 0.455 | 0.125 | 0.187 |
| 6 | 0.715 | 0.873 | 0.353 | 0.431 | 0.409 | 0.499 |
| 7 | 0.835 | 1 | 0.434 | 0.530 | 0.329 | 0.403 |
| 8 | 0.752 | 0.919 | 0.228 | 0.342 | 0.549 | 0.671 |
| 9 | 0.863 | 1 | 0.518 | 0.633 | 0.176 | 0.264 |

Table 8.3.5-2: UMidyn Spatial Correlation Pass/Fail Limits of the UMi Route with full path loss model [7] for 2450 MHz

| Segment # | SCF at Δd=22.9 mm =0.19 λ |  | SCF at Δd=90.5 mm =0.74 λ |  | SCF at Δd=208.0 mm =1.70 λ |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Lower | Upper | Lower | Upper | Lower | Upper |
| 1 | 0.850 | 1 | 0.680 | 0.831 | 0.660 | 0.806 |
| 2 | 0.883 | 1 | 0.842 | 1 | 0.843 | 1 |
| 3 | 0.708 | 0.866 | 0.177 | 0.265 | 0.130 | 0.194 |
| 4 | 0.688 | 0.840 | 0.263 | 0.395 | 0.256 | 0.384 |
| 5 | 0.770 | 0.942 | 0.519 | 0.635 | 0.230 | 0.346 |
| 6 | 0.876 | 1 | 0.728 | 0.890 | 0.616 | 0.752 |
| 7 | 0.884 | 1 | 0.856 | 1 | 0.849 | 1 |

8.3.6	Pass/Fail Criteria of Cross-polarization

This clause defines the pass/fail criteria of cross-polarization.

The cross-polarization ratio pass/fail limit is specified as ±2dB.

9	gNodeB configurations

9.1	General

This clause describes gNodeB emulator configurations for dynamic MIMO OTA testing, including FR1 common configuration parameters, specific configuration parameters for 4x4 FDD, 4x4 TDD, 2x2 FDD, and 2x2 TDD, as well as gNodeB emulator configurations for dynamic link adaptations.

9.2	gNodeB emulator settings

The gNB emulator used to execute tests at FR1 frequencies described in this document shall be configured according to the common configuration parameters in Table 9.2-1. The 4x4 FDD specific configuration are provided in Tables 9.2-2 and 9.2-3, while the 4x4 TDD specific configuration are provided in Tables 9.2-4 and 9.2-5. The proposed gNB parameters in are based on 3GPP MIMO OTA gNB configuration parameters in TS 38.151 [4] and the subsequent reference channels in TS 38.101-4 [8].

Table 9.2-1: FR1 Common test parameters for gNB emulator configuration

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
| PDSCH transmission scheme |  |  |  | Transmission scheme 1 |  |
| Carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 2) |  | RBs | 0 |  |
|  | Subcarrier spacing |  | kHz | 15, 30 |  |
| DL BWP configuration #1 | Cyclic prefix |  |  | Normal |  |
|  | RB offset |  | RBs | 0 |  |
|  | Number of contiguous PRB |  | PRBs | Maximum transmission bandwidth configuration as specified in clause 5.3.2 of TS 38.101-1 for tested channel bandwidth and subcarrier spacing |  |
| Common serving cell parameters | Physical Cell ID |  |  | 0 |  |
|  | SSB position in burst |  |  | First SSB in Slot #0 |  |
|  | SSB periodicity |  | ms | 20 |  |
|  | First DMRS position for Type A PDSCH mapping |  |  | 2 |  |
| PDCCH configuration | Slots for PDCCH monitoring |  |  | Each slot |  |
|  | Symbols with PDCCH |  | Symbols | 0, 1 |  |
|  | Number of PRBs in CORESET |  |  | Table 5.2-2 of TS 38.101-4 [8] for tested channel bandwidth and subcarrier spacing |  |
|  | Number of PDCCH candidates and aggregation levels |  |  | 1/AL8 |  |
|  | CCE-to-REG mapping type |  |  | Non-interleaved |  |
|  | DCI format |  |  | 1_1 |  |
|  | TCI state |  |  | TCI state #1 |  |
| Cross carrier scheduling |  |  |  | Not configured |  |
| CSI-RS for tracking | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resource 1,2,3,4 |  |
|  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 |  |
|  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 |  |
|  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |  |
|  | Density (ρ) |  |  | 3 for CSI-RS resource 1,2,3,4 |  |
|  | CSI-RS periodicity |  | Slots | 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 30 kHz SCS: 40 for CSI-RS resource 1,2,3,4 |  |
|  | CSI-RS offset |  | Slots | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |  |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |  |
|  | QCL info |  |  | TCI state #0 |  |
| NZP CSI-RS for CSI acquisition | First subcarrier index in the PRB used for CSI-RS |  |  | k0 = 0 |  |
|  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 12 |  |
|  | Number of CSI-RS ports (X) |  |  | Same as number of transmit antenna |  |
|  | CDM Type |  |  | ‘FD-CDM2’ |  |
|  | Density (ρ) |  |  | 1 |  |
|  | CSI-RS periodicity |  | Slots | 15 kHz SCS: 20 30 kHz SCS: 40 |  |
|  | CSI-RS offset |  | Slots | 0 |  |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |  |
|  | QCL info |  |  | TCI state #1 |  |
| ZP CSI-RS for CSI acquisition | First subcarrier index in the PRB used for CSI-RS |  |  | k0 = 4 |  |
|  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 12 |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |
|  | CDM Type |  |  | ‘FD-CDM2’ |  |
|  | Density (ρ) |  |  | 1 |  |
|  | CSI-RS periodicity |  | Slots | 15 kHz SCS: 20 30 kHz SCS: 40 |  |
|  | CSI-RS offset |  | Slots | 0 |  |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = BWP size |  |
| PDSCH DMRS configuration | Antenna ports indexes |  |  | {1000, 1001} for Rank 2 tests {1000-1003} for Rank 4 tests |  |
|  | Number of PDSCH DMRS CDM group(s) without data |  |  | 1 for Rank 2 tests 2 for Rank 4 tests |  |
| TCI state #0 | Type 1 QCL information | SSB index |  | SSB #0 |  |
|  |  | QCL Type |  | Type C |  |
|  | Type 2 QCL information | SSB index |  | N/A |  |
|  |  | QCL Type |  | N/A |  |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from ‘CSI-RS for tracking’ configuration |  |
|  |  | QCL Type |  | Type A |  |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |  |
|  |  | QCL Type |  | N/A |  |
| PT-RS configuration |  |  |  | PT-RS is not configured |  |
| Maximum number of code block groups for ACK/NACK feedback |  |  |  | 1 |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |
| HARQ ACK/NACK bundling |  |  |  | Multiplexed |  |
| Redundancy version coding sequence |  |  |  | N/A |  |
| Precoding configuration |  |  |  | SP Type I, Random per slot with PRB bundling granularity |  |
| Symbols for all unused REs |  |  |  | OCNG Annex A.5 of TS 38.101-4 [8] |  |
| Minimum Number of Slots per Stream |  |  |  | 20000 for 15kHz SCS 40000 for 30kHz SCS |
| Transmit Power Control |  |  | dBm | 13 dBm |
| Note 1: UE assumes that the TCI state for the PDSCH is identical to the TCI state applied for the PDCCH transmission. Note 2: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-1 for tested channel bandwidth and subcarrier spacing. |  |  |  |  |  |

Table 9.2-2: Test Parameters for FR1 4x4 FDD

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Modulation DL |  |  | CQI based |
| Modulation UL |  |  | QPSK |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |

Table 9.2-3: NR band specific parameters for FR1 4x4 FDD

| Parameters |  | 5G NR band |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | n2 | n5 | n12 | n14 | n25 | n26 | n30 | n66 | n70 | n71 |
| Bandwidth (MHz) |  | 10 | 10 | 10 | 10 | 20 | 10 | 10 | 20 | 15 | 10 |
| SCS |  | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 30 |
| Reference channel measurement | Number of allocated resource blocks | 52 | 52 | 52 | 52 | 106 | 52 | 52 | 106 | 79 | 24 |
|  | Number of consecutive PDSCH symbols | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
|  | Allocated slots per 2 frames | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 19 |
|  | Overhead for TBS determination | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Number of DMRS REs | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
|  | Number of MIMO layers | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |

Table 9.2-4: Test Parameters for FR1 4x4 TDD

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Modulation DL |  |  | CQI based/VRMC |
| Modulation UL |  |  | QPSK |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | Specific to the Reference channel |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 |
| TDD UL-DL pattern |  |  | FR1.30-1 as defined in Annex A.1.2 of TS 38.101-4 [8] for 30 kHz Table A.1.2-1: TDD UL-DL configuration for SCS 15 kHz |
| CSI-RS for tracking | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 |
|  | CSI-RS periodicity | Slots | 40 |
|  | CSI-RS offset | Slots | 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |

Table 9.2-5: NR band specific parameters for FR1 4x4 TDD

| Parameters |  | 5G NR band |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | n41 | n48 | n77 | n78 |
| Bandwidth (MHz) |  | 100 | 100 | 100 | 100 |
| SCS |  | 30 | 30 | 30 | 30 |
| Reference channel measurement | Allocated resource bocks | 273 | 273 | 273 | 273 |
|  | For 30 kHz For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | 4 | 4 | 4 | 4 |
|  | For 15 kHz For Slot i, if mod(i, 5) = 3 for i from {0,…,19} |  |  |  |  |
|  | For 15 kHz For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} |  |  |  |  |
|  | For 30 kHz For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | 12 | 12 | 12 | 12 |
|  | Allocated slots per 2 frames | 31 | 31 | 31 | 31 |
|  | Number of MIMO layers | 4 | 4 | 4 | 4 |
|  | Overhead for TBS determination | 0 | 0 | 0 | 0 |

The gNB emulator used to execute the Spatial Multiplexing (SM) tests at FR1 frequencies for 2x2 MIMO configuration described in this document shall be configured according to the common configuration parameters in Table 9.2-1. The FDD and TDD specific configurations for 2x2 FDD and 2x2 TDD tests are given in Table 9.2-6 and 9.2-8 respectively.  The NR band specific configurations are given in Tables 9.2-7 and Table 9.2-9 respectively.

Table 9.2-6: Test Parameters for FR1 2x2 FDD

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Modulation DL |  |  | CQI based |
| Modulation UL |  |  | QPSK |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |

Table 9.2-7: NR band specific parameters for FR1 2x2 FDD

| Parameters |  | 5G NR band |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | n2 | n5 | n12 | n14 | n25 | n26 | n30 | n66 | n70 | n71 |
| Bandwidth (MHz) |  | 10 | 10 | 10 | 10 | 20 | 10 | 10 | 20 | 15 | 10 |
| SCS |  | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 30 |
| Reference channel measurement | Number of allocated resource blocks | 52 | 52 | 52 | 52 | 106 | 52 | 52 | 106 | 79 | 24 |
|  | Number of consecutive PDSCH symbols | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
|  | Allocated slots per 2 frames | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 19 | 19 |
|  | Overhead for TBS determination | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Number of DMRS REs | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
|  | Number of MIMO layers | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |

Table 9.2-8: Test Parameters for FR1 2x2 TDD

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Modulation DL |  |  | CQI based |
| Modulation UL |  |  | QPSK |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | Specific to the Reference channel |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 |
| TDD UL-DL pattern |  |  | FR1.30-1 as defined in Annex A.1.2 of TS 38.101-4 [8] for 30 kHz Table A.1.2-1 of TS 38.101-4 [8]: TDD UL-DL configuration for SCS 15 kHz |

Table 9.2-9: NR band specific parameters for FR1 2x2 TDD

| Parameters |  | 5G NR band |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | n41 | n48 | n77 | n78 |
| Bandwidth (MHz) |  | 100 | 100 | 100 | 100 |
| SCS |  | 30 | 30 | 30 | 30 |
| Reference channel measurement | Allocated resource bocks | 273 | 273 | 273 | 273 |
|  | For 30 kHz For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | 4 | 4 | 4 | 4 |
|  | For 15 kHz For Slot i, if mod(i, 5) = 3 for i from {0,…,19} |  |  |  |  |
|  | For 15 kHz For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} |  |  |  |  |
|  | For 30 kHz For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} | 12 | 12 | 12 | 12 |
|  | Allocated slots per 2 frames | 31 | 31 | 31 | 31 |
|  | Number of MIMO layers | 2 | 2 | 2 | 2 |
|  | Overhead for TBS determination | 0 | 0 | 0 | 0 |

9.3	Dynamic link adaptation for MCS, rank and MIMO precoding

In the 5G NR dynamic scenario, the DUT is moving and hence link adaptation as well as the PDSCH MIMO precoding and beam alignment needs to be considered. The dynamic scheduling for MCS and rank adaptation follows Clauses 6.4.2 and 6.4.3 of [8] for 2RX and 4RX, where the gNB emulator adjusts the MCS and Rank based on the DUT’s reporting (CQI/RI reporting). The PDCCH DCI 1_1 and PDCCH DCI_1_0 transmitted by the gNB contains the link adaptation parameter for the DUT.

The PDSCH MIMO precoding and beam alignment by the gNB is configured by modifying the precoding matrix by the gNB based the PMI reporting from the DUT. Hence, the gNB emulator should adjust the MCS/RI/Precoding matrix based on the CQI/RI/PMI reporting from the DUT. The configuration for CSI configurations given in Table 9.3-1.

Table 9.3-1: CSI configuration

| Parameter |  | Unit | Test |
| --- | --- | --- | --- |
| TDD Slot Configuration (Note 1) |  |  | FR1.30-1 |
| SNR |  | dB | 22 |
| Beamforming Model |  |  | As defined in Clause B.4.1 in TS 38.101-4 [8] |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig-periodicity and offset | slot | 10/1 |
| ReportConfigType |  |  | Periodic |
| CQI-table (Note 3) |  |  | Table 2 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | not configured |
| timeRestrictionForInterferenceMeasurements |  |  | not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 16 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report periodicity and offset |  | slot | 10/9 |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1, CodebookConfig-N2) |  | 2-1 |
|  | CodebookSubsetRestriction |  | 11111111 |
|  | RI Restriction |  | 00001111 for follow RI |
| Physical channel for CSI report |  |  | PUCCH |
| CQI/RI/PMI delay (Note 2) |  | ms | 9.5 |
| Maximum number of HARQ transmission |  |  | FFS |
| Note 1: FR1.30-1 is defined in Clause A.1.2 of TS 38.101-4 [8] Note:2: Let CQI/RI/PMI delay corresponds to ‘k’ slots. Then, if the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#[(n-k/2)], this reported PMI cannot be applied at the gNB downlink before slot#[(n+k/2)]. Note 3: CQI table to use for CQI calculation (see clause 5.2.2.1 of TS 38.214 [9]) |  |  |  |

The gNB emulator used to execute tests at FR1 frequencies described in this document shall be configured according to the common configuration parameters

Annex A:
Channel model parameters

<Editor’s Note: The UMa parameter table A-1 is applicable to frequencies fc≤6 GHz per Table 7.5-6 of 38.901, specifically Note 6. The UMi parameter table A-2 is applicable to 2450 MHz only as per Note 7 from Table 7.5-6 of 38.901, UMi parameter tables above fc=2 GHz need to be defined per band. >

Table A-1: Parameter Table for Dynamic UMa Channel Model for 6 GHz and below

| Way point | 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Cluster# | Power [dB] | Excess delay [ns] | AoA [°] | AoD [°] | ASA [°] | ZoD [°] |
| LOS | -0.51 | 0 | 179.99 | -0.01 | 0 | 93.09 |
| 1 | -13.83 | 0 | 179.99 | -0.01 | 29.75 | 93.09 |
| 2 | -19.13 | 3 | -157.71 | 79.75 | 29.75 | 89.46 |
| 3 | -21.33 | 58 | -157.71 | 79.75 | 26.775 | 89.46 |
| 4 | -23.13 | 128 | -157.71 | 79.75 | 23.8 | 89.46 |
| 5 | -18.23 | 132 | 116.76 | 11.61 | 29.75 | 92.81 |
| 6 | -23.23 | 167 | -20.09 | 30.93 | 29.75 | 93.09 |
| 7 | -20.43 | 170 | 116.76 | 11.61 | 26.775 | 92.81 |
| 8 | -22.23 | 244 | 116.76 | 11.61 | 23.8 | 92.81 |
| 9 | -28.13 | 380 | 147.62 | -57.68 | 29.75 | 90.27 |
| 10 | -23.93 | 747 | -14.52 | -29.43 | 29.75 | 91.08 |
| 11 | -25.13 | 887 | 44.63 | 47.02 | 29.75 | 94.57 |
| 12 | -30.33 | 913 | 95.59 | -118.13 | 29.75 | 88 |
| 13 | -28.03 | 1178 | 177.77 | 69.02 | 29.75 | 89.74 |
| Ini. delay [ns] | XPR [dB] | PL [dB] | ZoA [°] | ASD [°] | ZSA [°] | ZSD [°] |
| 1456 | 11 | 94.69 | 90 | 4.47 | 0 | 0.84 |
| UE speed [m/s] | UE DoT Az [°] | UE coordinates (x,y,z) [m] | BS coordinates (x,y,z) [m] | K-factor [dB] |  |  |
| 8.33 | 125.17 | (435.78,-0.11,1.5) | (0,0,25) | 9 |  |  |
| Way point | 2 |  |  |  |  |  |
| Cluster# | Power [dB] | Excess delay [ns] | AoA [°] | AoD [°] | ASA [°] | ZoD [°] |
| LOS | -0.51 | 0 | -162.14 | 17.86 | 0 | 93.6 |
| 1 | -13.83 | 0 | -162.14 | 17.86 | 29.75 | 93.6 |
| 2 | -19.13 | 3 | -139.84 | 97.62 | 29.75 | 88.68 |
| 3 | -21.33 | 58 | -139.84 | 97.62 | 26.775 | 88.68 |
| 4 | -23.13 | 128 | -139.84 | 97.62 | 23.8 | 88.68 |
| 5 | -18.23 | 132 | 134.63 | 29.48 | 29.75 | 93.22 |
| 6 | -23.23 | 167 | -2.22 | 48.8 | 29.75 | 93.6 |
| 7 | -20.43 | 170 | 134.63 | 29.48 | 26.775 | 93.22 |
| 8 | -22.23 | 244 | 134.63 | 29.48 | 23.8 | 93.22 |
| 9 | -28.13 | 380 | 165.49 | -39.81 | 29.75 | 89.78 |
| 10 | -23.93 | 747 | 3.35 | -11.56 | 29.75 | 90.87 |
| 11 | -25.13 | 887 | 62.5 | 64.89 | 29.75 | 95.61 |
| 12 | -30.33 | 913 | 113.46 | -100.26 | 29.75 | 86.71 |
| 13 | -28.03 | 1178 | -164.36 | 86.89 | 29.75 | 89.06 |
| Ini. delay [ns] | XPR [dB] | PL [dB] | ZoA [°] | ASD [°] | ZSA [°] | ZSD [°] |
| 1247 | 11 | 92.38 | 90 | 4.47 | 0 | 1.14 |
| UE speed [m/s] | UE DoT Az [°] | UE coordinates (x,y,z) [m] | BS coordinates (x,y,z) [m] | K-factor [dB] |  |  |
| 8.33 | 112.77 | (355.11,114.39,1.5) | (0,0,25) | 9 |  |  |
| Way point | 3 |  |  |  |  |  |
| Cluster# | Power [dB] | Excess delay [ns] | AoA [°] | AoD [°] | ASA [°] | ZoD [°] |
| 1 | -12.09 | 0 | -77.57 | -18.59 | 15.63 | 102.25 |
| 2 | -8.89 | 76 | 137.57 | -2.91 | 15.63 | 102.71 |
| 3 | -10.19 | 79 | -105.19 | -14.7 | 15.63 | 103.35 |
| 4 | -11.19 | 81 | 137.57 | -2.91 | 14.067 | 102.71 |
| 5 | -12.89 | 85 | 137.57 | -2.91 | 12.504 | 102.71 |
| 6 | -7.69 | 232 | -169.91 | 12.32 | 15.63 | 102.9 |
| 7 | -9.89 | 235 | -169.91 | 12.32 | 14.067 | 102.9 |
| 8 | -11.59 | 239 | -169.91 | 12.32 | 12.504 | 102.9 |
| 9 | -15.09 | 240 | 70.25 | 60.29 | 15.63 | 104.83 |
| 10 | -14.79 | 289 | 81.82 | -30.38 | 15.63 | 101.64 |
| 11 | -18.39 | 299 | -22.44 | 64.97 | 15.63 | 105.12 |
| 12 | -18.79 | 340 | 61.39 | -51.87 | 15.63 | 101.06 |
| 13 | -12.79 | 447 | 83.48 | -24.32 | 15.63 | 104.35 |
| 14 | -14.49 | 476 | -43.91 | -30.25 | 15.63 | 104.51 |
| 15 | -16.39 | 790 | 97.45 | -39.61 | 15.63 | 100.9 |
| 16 | -20.89 | 987 | 44.51 | 79.79 | 15.63 | 104.51 |
| 17 | -21.59 | 1551 | -4.58 | 77.49 | 15.63 | 101.51 |
| 18 | -21.59 | 1675 | 16.47 | 70.63 | 15.63 | 100.93 |
| 19 | -23.49 | 1999 | -1.76 | -55.03 | 15.63 | 100.64 |
| 20 | -24.79 | 2042 | 22.62 | 72.87 | 15.63 | 105.32 |
| 21 | -23.69 | 2296 | 18.35 | 73.6 | 15.63 | 100.9 |
| 22 | -23.39 | 2417 | 13.24 | 82.36 | 15.63 | 100.87 |
| 23 | -29.29 | 2564 | -10.31 | 90.86 | 15.63 | 104.83 |
| 24 | -30.49 | 3151 | 47.53 | -69.46 | 15.63 | 105.67 |
| Ini. delay [ns] | XPR [dB] | PL [dB] | ZoA [°] | ASD [°] | ZSA [°] | ZSD [°] |
| 1245 | 7 | 121.84 | 90 | 1.32 | 0 | 0.97 |
| UE speed [m/s] | UE DoT Az [°] | UE coordinates (x,y,z) [m] | BS coordinates (x,y,z) [m] | K-factor [dB] |  |  |
| 0.833 | 161.92 | (352.65,120.25,1.5) | (0,0,25) | - |  |  |
| Way point | 4 |  |  |  |  |  |
| Cluster# | Power [dB] | Excess delay [ns] | AoA [°] | AoD [°] | ASA [°] | ZoD [°] |
| 1 | -18.8 | 0 | 91.07 | -42.7 | 9.56 | 98.71 |
| 2 | -5.4 | 139 | -133.34 | 18.09 | 9.56 | 100.85 |
| 3 | -7.6 | 147 | -133.34 | 18.09 | 8.604 | 100.85 |
| 4 | -11.4 | 168 | 113.06 | 51.08 | 9.56 | 102.28 |
| 5 | -13.6 | 196 | 113.06 | 51.08 | 8.604 | 102.28 |
| 6 | -15.9 | 209 | -2.18 | 62.03 | 9.56 | 103.69 |
| 7 | -9.4 | 214 | -133.34 | 18.09 | 7.648 | 100.85 |
| 8 | -15.3 | 244 | 113.06 | 51.08 | 7.648 | 102.28 |
| 9 | -12.9 | 277 | -37.03 | -9 | 9.56 | 98.96 |
| 10 | -21.3 | 560 | 128.35 | 74.92 | 9.56 | 97.52 |
| 11 | -12 | 691 | 91.59 | -9.46 | 9.56 | 102.5 |
| 12 | -17.8 | 791 | 69.6 | -33.93 | 9.56 | 103.75 |
| 13 | -22.1 | 810 | -101.35 | 66.67 | 9.56 | 104.75 |
| 14 | -20.6 | 908 | 113.06 | -40.57 | 9.56 | 104.04 |
| 15 | -16.2 | 915 | -6.7 | -25.85 | 9.56 | 98.56 |
| 16 | -16.7 | 1114 | -20.61 | -27.99 | 9.56 | 98.22 |
| 17 | -18.1 | 1486 | -41.64 | 77.37 | 9.56 | 98.37 |
| 18 | -21.6 | 1623 | 142.43 | 71.43 | 9.56 | 104.27 |
| 19 | -23.7 | 1664 | 172.07 | 65.94 | 9.56 | 96.75 |
| 20 | -24.3 | 1747 | -178.54 | -21.9 | 9.56 | 97.04 |
| 21 | -22 | 1823 | 135.13 | -34.31 | 9.56 | 104.76 |
| 22 | -25.3 | 1931 | -132.56 | 63.77 | 9.56 | 97.34 |
| 23 | -35.1 | 3517 | 94.46 | -0.09 | 9.56 | 103.42 |
| Ini. delay [ns] | XPR [dB] | PL [dB] | ZoA [°] | ASD [°] | ZSA [°] | ZSD [°] |
| 1189 | 10 | 121.06 | 90 | 1.75 | 0 | 0.15 |
| UE speed [m/s] | UE DoT Az [°] | UE coordinates (x,y,z) [m] | BS coordinates (x,y,z) [m] | K-factor [dB] |  |  |
| 0.833 | 155.87 | (332.43,126.85,1.5) | (0,0,25) | - |  |  |
| Way point | 5 |  |  |  |  |  |
| Cluster# | Power [dB] | Excess delay [ns] | AoA [°] | AoD [°] | ASA [°] | ZoD [°] |
| 1 | -18.8 | 0 | 92.44 | -41.33 | 9.56 | 98.7 |
| 2 | -5.4 | 139 | -131.97 | 19.46 | 9.56 | 100.93 |
| 3 | -7.6 | 147 | -131.97 | 19.46 | 8.604 | 100.93 |
| 4 | -11.4 | 168 | 114.43 | 52.45 | 9.56 | 102.42 |
| 5 | -13.6 | 196 | 114.43 | 52.45 | 8.604 | 102.42 |
| 6 | -15.9 | 209 | -0.81 | 63.4 | 9.56 | 103.89 |
| 7 | -9.4 | 214 | -131.97 | 19.46 | 7.648 | 100.93 |
| 8 | -15.3 | 244 | 114.43 | 52.45 | 7.648 | 102.42 |
| 9 | -12.9 | 277 | -35.66 | -7.63 | 9.56 | 98.96 |
| 10 | -21.3 | 560 | 129.72 | 76.29 | 9.56 | 97.47 |
| 11 | -12 | 691 | 92.96 | -8.09 | 9.56 | 102.65 |
| 12 | -17.8 | 791 | 70.97 | -32.56 | 9.56 | 103.95 |
| 13 | -22.1 | 810 | -99.98 | 68.04 | 9.56 | 104.99 |
| 14 | -20.6 | 908 | 114.43 | -39.2 | 9.56 | 104.25 |
| 15 | -16.2 | 915 | -5.33 | -24.48 | 9.56 | 98.55 |
| 16 | -16.7 | 1114 | -19.24 | -26.62 | 9.56 | 98.2 |
| 17 | -18.1 | 1486 | -40.27 | 78.74 | 9.56 | 98.35 |
| 18 | -21.6 | 1623 | 143.8 | 72.8 | 9.56 | 104.49 |
| 19 | -23.7 | 1664 | 173.44 | 67.31 | 9.56 | 96.66 |
| 20 | -24.3 | 1747 | -177.17 | -20.53 | 9.56 | 96.97 |
| 21 | -22 | 1823 | 136.5 | -32.94 | 9.56 | 105 |
| 22 | -25.3 | 1931 | -131.19 | 65.14 | 9.56 | 97.28 |
| 23 | -35.1 | 3517 | 95.83 | 1.28 | 9.56 | 103.61 |
| Ini. delay [ns] | XPR [dB] | PL [dB] | ZoA [°] | ASD [°] | ZSA [°] | ZSD [°] |
| 1162 | 10 | 120.67 | 90 | 1.75 | 0 | 0.16 |
| UE speed [m/s] | UE DoT Az [°] | UE coordinates (x,y,z) [m] | BS coordinates (x,y,z) [m] | K-factor [dB] |  |  |
| 8.33 | -130.59 | (321.65,131.68,1.5) | (0,0,25) | - |  |  |
| Way point | 6 |  |  |  |  |  |
| Cluster# | Power [dB] | Excess delay [ns] | AoA [°] | AoD [°] | ASA [°] | ZoD [°] |
| 1 | -12.09 | 0 | -96.5 | -37.52 | 15.63 | 104.24 |
| 2 | -8.89 | 76 | 118.64 | -21.84 | 15.63 | 105.24 |
| 3 | -10.19 | 79 | -124.12 | -33.63 | 15.63 | 106.67 |
| 4 | -11.19 | 81 | 118.64 | -21.84 | 14.067 | 105.24 |
| 5 | -12.89 | 85 | 118.64 | -21.84 | 12.504 | 105.24 |
| 6 | -7.69 | 232 | 171.16 | -6.61 | 15.63 | 105.67 |
| 7 | -9.89 | 235 | 171.16 | -6.61 | 14.067 | 105.67 |
| 8 | -11.59 | 239 | 171.16 | -6.61 | 12.504 | 105.67 |
| 9 | -15.09 | 240 | 51.32 | 41.36 | 15.63 | 109.94 |
| 10 | -14.79 | 289 | 62.89 | -49.31 | 15.63 | 102.89 |
| 11 | -18.39 | 299 | -41.37 | 46.04 | 15.63 | 110.59 |
| 12 | -18.79 | 340 | 42.46 | -70.8 | 15.63 | 101.61 |
| 13 | -12.79 | 447 | 64.55 | -43.25 | 15.63 | 108.87 |
| 14 | -14.49 | 476 | -62.84 | -49.18 | 15.63 | 109.23 |
| 15 | -16.39 | 790 | 78.52 | -58.54 | 15.63 | 101.25 |
| 16 | -20.89 | 987 | 25.58 | 60.86 | 15.63 | 109.23 |
| 17 | -21.59 | 1551 | -23.51 | 58.56 | 15.63 | 102.6 |
| 18 | -21.59 | 1675 | -2.46 | 51.7 | 15.63 | 101.32 |
| 19 | -23.49 | 1999 | -20.69 | -73.96 | 15.63 | 100.68 |
| 20 | -24.79 | 2042 | 3.69 | 53.94 | 15.63 | 111.01 |
| 21 | -23.69 | 2296 | -0.58 | 54.67 | 15.63 | 101.25 |
| 22 | -23.39 | 2417 | -5.69 | 63.43 | 15.63 | 101.18 |
| 23 | -29.29 | 2564 | -29.24 | 71.93 | 15.63 | 109.94 |
| 24 | -30.49 | 3151 | 28.6 | -88.39 | 15.63 | 111.8 |
| Ini. delay [ns] | XPR [dB] | PL [dB] | ZoA [°] | ASD [°] | ZSA [°] | ZSD [°] |
| 700 | 7 | 112.06 | 90 | 1.32 | 0 | 2.14 |
| UE speed [m/s] | UE DoT Az [°] | UE coordinates (x,y,z) [m] | BS coordinates (x,y,z) [m] | K-factor [dB] |  |  |
| 8.33 | -51.25 | (208.50,-0.36,1.5) | (0,0,25) | - |  |  |
| Way point | 7 |  |  |  |  |  |
| Cluster# | Power [dB] | Excess delay [ns] | AoA [°] | AoD [°] | ASA [°] | ZoD [°] |
| 1 | -12.09 | 0 | -119.5 | -60.52 | 15.63 | 102.45 |
| 2 | -8.89 | 76 | 95.64 | -44.84 | 15.63 | 102.97 |
| 3 | -10.19 | 79 | -147.12 | -56.63 | 15.63 | 103.71 |
| 4 | -11.19 | 81 | 95.64 | -44.84 | 14.067 | 102.97 |
| 5 | -12.89 | 85 | 95.64 | -44.84 | 12.504 | 102.97 |
| 6 | -7.69 | 232 | 148.16 | -29.61 | 15.63 | 103.19 |
| 7 | -9.89 | 235 | 148.16 | -29.61 | 14.067 | 103.19 |
| 8 | -11.59 | 239 | 148.16 | -29.61 | 12.504 | 103.19 |
| 9 | -15.09 | 240 | 28.32 | 18.36 | 15.63 | 105.41 |
| 10 | -14.79 | 289 | 39.89 | -72.31 | 15.63 | 101.75 |
| 11 | -18.39 | 299 | -64.37 | 23.04 | 15.63 | 105.74 |
| 12 | -18.79 | 340 | 19.46 | -93.8 | 15.63 | 101.09 |
| 13 | -12.79 | 447 | 41.55 | -66.25 | 15.63 | 104.86 |
| 14 | -14.49 | 476 | -85.84 | -72.18 | 15.63 | 105.04 |
| 15 | -16.39 | 790 | 55.52 | -81.54 | 15.63 | 100.9 |
| 16 | -20.89 | 987 | 2.58 | 37.86 | 15.63 | 105.04 |
| 17 | -21.59 | 1551 | -46.51 | 35.56 | 15.63 | 101.6 |
| 18 | -21.59 | 1675 | -25.46 | 28.7 | 15.63 | 100.94 |
| 19 | -23.49 | 1999 | -43.69 | -96.96 | 15.63 | 100.6 |
| 20 | -24.79 | 2042 | -19.31 | 30.94 | 15.63 | 105.97 |
| 21 | -23.69 | 2296 | -23.58 | 31.67 | 15.63 | 100.9 |
| 22 | -23.39 | 2417 | -28.69 | 40.43 | 15.63 | 100.86 |
| 23 | -29.29 | 2564 | -52.24 | 48.93 | 15.63 | 105.41 |
| 24 | -30.49 | 3151 | 5.6 | -111.39 | 15.63 | 106.37 |
| Ini. delay [ns] | XPR [dB] | PL [dB] | ZoA [°] | ASD [°] | ZSA [°] | ZSD [°] |
| 1151 | 7 | 120.5 | 90 | 1.32 | 0 | 1.11 |
| UE speed [m/s] | UE DoT Az [°] | UE coordinates (x,y,z) [m] | BS coordinates (x,y,z) [m] | K-factor [dB] |  |  |
| 8.33 | 45.03 | (316.65,-135.09,1.5) | (0,0,25) | - |  |  |
| Way point | 8 |  |  |  |  |  |
| Cluster# | Power [dB] | Excess delay [ns] | AoA [°] | AoD [°] | ASA [°] | ZoD [°] |
| LOS | -0.51 | 0 | 158.87 | -21.13 | 0 | 93.85 |
| 1 | -22.24 | 0 | 158.87 | -21.13 | 18.91 | 93.85 |
| 2 | -16.01 | 47 | -119.23 | 40.14 | 18.91 | 97.13 |
| 3 | -18.31 | 49 | -119.23 | 40.14 | 17.019 | 97.13 |
| 4 | -23.11 | 49 | 24.46 | -42.55 | 18.91 | 93.71 |
| 5 | -20.01 | 51 | -119.23 | 40.14 | 15.128 | 97.13 |
| 6 | -22.61 | 65 | 43.54 | -3.87 | 18.91 | 94.71 |
| 7 | -18.81 | 173 | -159.02 | -11.22 | 18.91 | 93.28 |
| 8 | -21.01 | 175 | -159.02 | -11.22 | 17.019 | 93.28 |
| 9 | -22.81 | 178 | -159.02 | -11.22 | 15.128 | 93.28 |
| 10 | -22.51 | 240 | -138.05 | -0.88 | 18.91 | 94.71 |
| 11 | -25.81 | 337 | -54.52 | 13.72 | 18.91 | 91.57 |
| 12 | -20.41 | 495 | 102.15 | -20.6 | 18.91 | 93.35 |
| 13 | -30.01 | 1091 | 46.04 | 38.44 | 18.91 | 91 |
| 14 | -29.41 | 1876 | 63.57 | 40.25 | 18.91 | 97.41 |
| Ini. delay [ns] | XPR [dB] | PL [dB] | ZoA [°] | ASD [°] | ZSA [°] | ZSD [°] |
| 1168 | 8 | 91. 76 | 90 | 5.33 | 0 | 2.14 |
| UE speed [m/s] | UE DoT Az [°] | UE coordinates (x,y,z) [m] | BS coordinates (x,y,z) [m] | K-factor [dB] |  |  |
| 8.33 | 5.64 | (325.81,-125.92,1.5) | (0,0,25) | 9 |  |  |
| Way point | 9 |  |  |  |  |  |
| Cluster# | Power [dB] | Excess delay [ns] | AoA [°] | AoD [°] | ASA [°] | ZoD [°] |
| LOS | -0.51 | 0 | 163.96 | -16.04 | 0 | 93.16 |
| 1 | -22.24 | 0 | 163.96 | -16.04 | 18.91 | 93.16 |
| 2 | -16.01 | 47 | -114.14 | 45.23 | 18.91 | 95.42 |
| 3 | -18.31 | 49 | -114.14 | 45.23 | 17.019 | 95.42 |
| 4 | -23.11 | 49 | 29.55 | -37.46 | 18.91 | 93.06 |
| 5 | -20.01 | 51 | -114.14 | 45.23 | 15.128 | 95.42 |
| 6 | -22.61 | 65 | 48.63 | 1.22 | 18.91 | 93.75 |
| 7 | -18.81 | 173 | -153.93 | -6.13 | 18.91 | 92.77 |
| 8 | -21.01 | 175 | -153.93 | -6.13 | 17.019 | 92.77 |
| 9 | -22.81 | 178 | -153.93 | -6.13 | 15.128 | 92.77 |
| 10 | -22.51 | 240 | -132.96 | 4.21 | 18.91 | 93.75 |
| 11 | -25.81 | 337 | -49.43 | 18.81 | 18.91 | 91.58 |
| 12 | -20.41 | 495 | 107.24 | -15.51 | 18.91 | 92.82 |
| 13 | -30.01 | 1091 | 51.13 | 43.53 | 18.91 | 91.19 |
| 14 | -29.41 | 1876 | 68.66 | 45.34 | 18.91 | 95.62 |
| Ini. delay [ns] | XPR [dB] | PL [dB] | ZoA [°] | ASD [°] | ZSA [°] | ZSD [°] |
| 1422 | 8 | 94.28 | 90 | 5.33 | 0 | 1.48 |
| UE speed [m/s] | UE DoT Az [°] | UE coordinates (x,y,z) [m] | BS coordinates (x,y,z) [m] | K-factor [dB] |  |  |
| 8.33 | 142.83 | (409.22,-117.68,1.5) | (0,0,25) | 9 |  |  |
| Way point | 10 |  |  |  |  |  |
| Cluster# | Power [dB] | Excess delay [ns] | AoA [°] | AoD [°] | ASA [°] | ZoD [°] |
| 1 | -18.8 | 0 | 56.01 | -77.76 | 9.56 | 98.73 |
| 2 | -5.4 | 139 | -168.4 | -16.97 | 9.56 | 100.52 |
| 3 | -7.6 | 147 | -168.4 | -16.97 | 8.604 | 100.52 |
| 4 | -11.4 | 168 | 78 | 16.02 | 9.56 | 101.72 |
| 5 | -13.6 | 196 | 78 | 16.02 | 8.604 | 101.72 |
| 6 | -15.9 | 209 | -37.24 | 26.97 | 9.56 | 102.9 |
| 7 | -9.4 | 214 | -168.4 | -16.97 | 7.648 | 100.52 |
| 8 | -15.3 | 244 | 78 | 16.02 | 7.648 | 101.72 |
| 9 | -12.9 | 277 | -72.09 | -44.06 | 9.56 | 98.94 |
| 10 | -21.3 | 560 | 93.29 | 39.86 | 9.56 | 97.74 |
| 11 | -12 | 691 | 56.53 | -44.52 | 9.56 | 101.91 |
| 12 | -17.8 | 791 | 34.54 | -68.99 | 9.56 | 102.95 |
| 13 | -22.1 | 810 | -136.41 | 31.61 | 9.56 | 103.79 |
| 14 | -20.6 | 908 | 78 | -75.63 | 9.56 | 103.19 |
| 15 | -16.2 | 915 | -41.76 | -60.91 | 9.56 | 98.61 |
| 16 | -16.7 | 1114 | -55.67 | -63.05 | 9.56 | 98.33 |
| 17 | -18.1 | 1486 | -76.7 | 42.31 | 9.56 | 98.45 |
| 18 | -21.6 | 1623 | 107.37 | 36.37 | 9.56 | 103.38 |
| 19 | -23.7 | 1664 | 137.01 | 30.88 | 9.56 | 97.09 |
| 20 | -24.3 | 1747 | 146.4 | -56.96 | 9.56 | 97.34 |
| 21 | -22 | 1823 | 100.07 | -69.37 | 9.56 | 103.79 |
| 22 | -25.3 | 1931 | -167.62 | 28.71 | 9.56 | 97.59 |
| 23 | -35.1 | 3517 | 59.4 | -35.15 | 9.56 | 102.68 |
| Ini. delay [ns] | XPR [dB] | PL [dB] | ZoA [°] | ASD [°] | ZSA [°] | ZSD [°] |
| 1312 | 10 | 122.73 | 90 | 1.75 | 0 | 0.12 |
| UE speed [m/s] | UE DoT Az [°] | UE coordinates (x,y,z) [m] | BS coordinates (x,y,z) [m] | K-factor [dB] |  |  |
| 0.833 | 34.08 | (380.81,-96.14,1.5) | (0,0,25) | - |  |  |
| Way point | 11 |  |  |  |  |  |
| Cluster# | Power [dB] | Excess delay [ns] | AoA [°] | AoD [°] | ASA [°] | ZoD [°] |
| LOS | -0.51 | 0 | 167.81 | -12.19 | 0 | 93.32 |
| 1 | -13.83 | 0 | 167.81 | -12.19 | 29.75 | 93.32 |
| 2 | -19.13 | 3 | -169.89 | 67.57 | 29.75 | 89.11 |
| 3 | -21.33 | 58 | -169.89 | 67.57 | 26.775 | 89.11 |
| 4 | -23.13 | 128 | -169.89 | 67.57 | 23.8 | 89.11 |
| 5 | -18.23 | 132 | 104.58 | -0.57 | 29.75 | 93 |
| 6 | -23.23 | 167 | -32.27 | 18.75 | 29.75 | 93.32 |
| 7 | -20.43 | 170 | 104.58 | -0.57 | 26.775 | 93 |
| 8 | -22.23 | 244 | 104.58 | -0.57 | 23.8 | 93 |
| 9 | -28.13 | 380 | 135.44 | -69.86 | 29.75 | 90.05 |
| 10 | -23.93 | 747 | -26.7 | -41.61 | 29.75 | 90.99 |
| 11 | -25.13 | 887 | 32.45 | 34.84 | 29.75 | 95.03 |
| 12 | -30.33 | 913 | 83.41 | -130.31 | 29.75 | 87.43 |
| 13 | -28.03 | 1178 | 165.59 | 56.84 | 29.75 | 89.44 |
| Ini. delay [ns] | XPR [dB] | PL [dB] | ZoA [°] | ASD [°] | ZSA [°] | ZSD [°] |
| 1355 | 11 | 93.44 | 90 | 4.47 | 0 | 0.97 |
| UE speed [m/s] | UE DoT Az [°] | UE coordinates (x,y,z) [m] | BS coordinates (x,y,z) [m] | K-factor [dB] |  |  |
| 8.33 | 65.26 | (396.39,-85.60,1.5) | (0,0,25) | 9 |  |  |

Table A-2: Parameter Table for Dynamic UMi Channel Model for 2450 MHz

| Way point | 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Cluster# | Power [dB] | Excess delay [ns] | AoA [°] | AoD [°] | ASA [°] | ZoD [°] |
| 1 | -12.09 | 0 | -120.32 | -37.16 | 12.29 | 100.34 |
| 2 | -8.89 | 23 | 125.38 | -21.64 | 12.29 | 100.44 |
| 3 | -10.19 | 24 | -142.11 | -33.31 | 12.29 | 100.6 |
| 4 | -11.19 | 24 | 125.38 | -21.64 | 11.061 | 100.44 |
| 5 | -12.89 | 26 | 125.38 | -21.64 | 9.832 | 100.44 |
| 6 | -7.69 | 70 | 166.82 | -6.59 | 12.29 | 100.49 |
| 7 | -9.89 | 71 | 166.82 | -6.59 | 11.061 | 100.49 |
| 8 | -11.59 | 72 | 166.82 | -6.59 | 9.832 | 100.49 |
| 9 | -15.09 | 72 | 72.27 | 40.87 | 12.29 | 100.96 |
| 10 | -14.79 | 87 | 81.39 | -48.83 | 12.29 | 100.19 |
| 11 | -18.39 | 90 | -76.83 | 45.5 | 12.29 | 101.03 |
| 12 | -18.79 | 103 | 65.28 | -70.08 | 12.29 | 100.05 |
| 13 | -12.79 | 135 | 82.71 | -42.83 | 12.29 | 100.84 |
| 14 | -14.49 | 144 | -93.77 | -48.7 | 12.29 | 100.88 |
| 15 | -16.39 | 238 | 93.73 | -57.96 | 12.29 | 100.01 |
| 16 | -20.89 | 298 | 51.96 | 60.17 | 12.29 | 100.88 |
| 17 | -21.59 | 468 | 13.23 | 57.89 | 12.29 | 100.16 |
| 18 | -21.59 | 505 | 29.84 | 51.11 | 12.29 | 100.02 |
| 19 | -23.49 | 603 | 15.45 | -73.21 | 12.29 | 99.95 |
| 20 | -24.79 | 616 | 34.69 | 53.32 | 12.29 | 101.07 |
| 21 | -23.69 | 693 | 31.32 | 54.04 | 12.29 | 100.01 |
| 22 | -23.39 | 729 | 27.29 | 62.71 | 12.29 | 100 |
| 23 | -29.29 | 774 | 8.71 | 71.12 | 12.29 | 100.96 |
| 24 | -30.49 | 951 | 54.34 | -87.49 | 12.29 | 101.16 |
| Ini. delay [ns] | XPR [dB] | PL [dB] | ZoA [°] | ASD [°] | ZSA [°] | ZSD [°] |
| 1389 | 7 | 123.16 | 90 | 1.3 | 0 | 0.23 |
| UE speed [m/s] | UE DoT Az [°] | UE coordinates (x,y,z) [m] | BS coordinates (x,y,z) [m] | K-factor [dB] |  |  |
| 0.833 | -39.55 | (416.41,0.53,1.5) | (0,0,10) | - |  |  |
| Way point | 2 |  |  |  |  |  |
| Cluster# | Power [dB] | Excess delay [ns] | AoA [°] | AoD [°] | ASA [°] | ZoD [°] |
| LOS | -0.51 | 0 | 178.36 | -1.64 | 0 | 91.13 |
| 1 | -13.83 | 0 | 178.36 | -1.64 | 22.34 | 91.13 |
| 2 | -19.13 | 2 | -76.03 | 85.1 | 22.34 | 87.85 |
| 3 | -21.33 | 33 | -76.03 | 85.1 | 20.106 | 87.85 |
| 4 | -23.13 | 74 | -76.03 | 85.1 | 17.872 | 87.85 |
| 5 | -18.23 | 76 | 130.73 | 11 | 22.34 | 90.88 |
| 6 | -23.23 | 96 | -61.17 | 32 | 22.34 | 91.13 |
| 7 | -20.43 | 98 | 130.73 | 11 | 20.106 | 90.88 |
| 8 | -22.23 | 141 | 130.73 | 11 | 17.872 | 90.88 |
| 9 | -28.13 | 219 | -117.21 | -64.36 | 22.34 | 88.58 |
| 10 | -23.93 | 431 | 31.83 | -33.63 | 22.34 | 89.32 |
| 11 | -25.13 | 511 | -12.42 | 49.51 | 22.34 | 92.47 |
| 12 | -30.33 | 527 | -62.84 | -130.09 | 22.34 | 86.54 |
| 13 | -28.03 | 679 | 87.88 | 73.43 | 22.34 | 88.11 |
| Ini. delay [ns] | XPR [dB] | PL [dB] | ZoA [°] | ASD [°] | ZSA [°] | ZSD [°] |
| 1442 | 11 | 104.42 | 90 | 4.86 | 0 | 0.76 |
| UE speed [m/s] | UE DoT Az [°] | UE coordinates (x,y,z) [m] | BS coordinates (x,y,z) [m] | K-factor [dB] |  |  |
| 8.33 | -123.94 | (432.02,-12.36,1.5) | (0,0,10) | 9 |  |  |
| Way point | 3 |  |  |  |  |  |
| Cluster# | Power [dB] | Excess delay [ns] | AoA [°] | AoD [°] | ASA [°] | ZoD [°] |
| LOS | -0.51 | 0 | 155.21 | -24.79 | 0 | 91.32 |
| 1 | -13.83 | 0 | 155.21 | -24.79 | 22.34 | 91.32 |
| 2 | -19.13 | 2 | -99.18 | 61.95 | 22.34 | 88.04 |
| 3 | -21.33 | 33 | -99.18 | 61.95 | 20.106 | 88.04 |
| 4 | -23.13 | 74 | -99.18 | 61.95 | 17.872 | 88.04 |
| 5 | -18.23 | 76 | 107.58 | -12.15 | 22.34 | 91.07 |
| 6 | -23.23 | 96 | -84.32 | 8.85 | 22.34 | 91.32 |
| 7 | -20.43 | 98 | 107.58 | -12.15 | 20.106 | 91.07 |
| 8 | -22.23 | 141 | 107.58 | -12.15 | 17.872 | 91.07 |
| 9 | -28.13 | 219 | -140.36 | -87.51 | 22.34 | 88.77 |
| 10 | -23.93 | 431 | 8.68 | -56.78 | 22.34 | 89.51 |
| 11 | -25.13 | 511 | -35.57 | 26.36 | 22.34 | 92.66 |
| 12 | -30.33 | 527 | -85.99 | -153.24 | 22.34 | 86.73 |
| 13 | -28.03 | 679 | 64.73 | 50.28 | 22.34 | 88.3 |
| Ini. delay [ns] | XPR [dB] | PL [dB] | ZoA [°] | ASD [°] | ZSA [°] | ZSD [°] |
| 1235 | 11 | 101.72 | 90 | 4.86 | 0 | 0.76 |
| UE speed [m/s] | UE DoT Az [°] | UE coordinates (x,y,z) [m] | BS coordinates (x,y,z) [m] | K-factor [dB] |  |  |
| 8.33 | -174.45 | (335.93,-155.15,1.5) | (0,0,10) | 9 |  |  |
| Way point | 4 |  |  |  |  |  |
| Cluster# | Power [dB] | Excess delay [ns] | AoA [°] | AoD [°] | ASA [°] | ZoD [°] |
| 1 | -18.8 | 0 | 70.85 | -88.72 | 7.52 | 98.1 |
| 2 | -5.4 | 42 | 177.83 | -28.59 | 7.52 | 98.58 |
| 3 | -7.6 | 44 | 177.83 | -28.59 | 6.768 | 98.58 |
| 4 | -11.4 | 51 | 88.2 | 4.06 | 7.52 | 98.9 |
| 5 | -13.6 | 59 | 88.2 | 4.06 | 6.768 | 98.9 |
| 6 | -15.9 | 63 | -78.68 | 14.88 | 7.52 | 99.21 |
| 7 | -9.4 | 64 | 177.83 | -28.59 | 6.016 | 98.58 |
| 8 | -15.3 | 74 | 88.2 | 4.06 | 6.016 | 98.9 |
| 9 | -12.9 | 84 | -106.18 | -55.39 | 7.52 | 98.16 |
| 10 | -21.3 | 169 | 100.27 | 27.64 | 7.52 | 97.84 |
| 11 | -12 | 209 | 71.26 | -55.84 | 7.52 | 98.95 |
| 12 | -17.8 | 239 | 53.91 | -80.04 | 7.52 | 99.22 |
| 13 | -22.1 | 244 | -156.93 | 19.48 | 7.52 | 99.45 |
| 14 | -20.6 | 274 | 88.2 | -86.61 | 7.52 | 99.29 |
| 15 | -16.2 | 276 | -82.25 | -72.05 | 7.52 | 98.07 |
| 16 | -16.7 | 336 | -93.22 | -74.16 | 7.52 | 97.99 |
| 17 | -18.1 | 448 | -109.82 | 30.06 | 7.52 | 98.03 |
| 18 | -21.6 | 490 | 111.38 | 24.19 | 7.52 | 99.34 |
| 19 | -23.7 | 502 | 134.77 | 18.76 | 7.52 | 97.67 |
| 20 | -24.3 | 527 | 142.17 | -68.15 | 7.52 | 97.73 |
| 21 | -22 | 550 | 105.62 | -80.42 | 7.52 | 99.45 |
| 22 | -25.3 | 583 | 178.45 | 16.61 | 7.52 | 97.8 |
| 23 | -35.1 | 1061 | 73.52 | -46.57 | 7.52 | 99.15 |
| Ini. delay [ns] | XPR [dB] | PL [dB] | ZoA [°] | ASD [°] | ZSA [°] | ZSD [°] |
| 1199 | 10 | 120.9 | 90 | 1.73 | 0 | 0.03 |
| UE speed [m/s] | UE DoT Az [°] | UE coordinates (x,y,z) [m] | BS coordinates (x,y,z) [m] | K-factor [dB] |  |  |
| 8.33 | 120.08 | (323.58,-156.35,1.5) | (0,0,10) | - |  |  |
| Way point | 5 |  |  |  |  |  |
| Cluster# | Power [dB] | Excess delay [ns] | AoA [°] | AoD [°] | ASA [°] | ZoD [°] |
| 1 | -12.09 | 0 | -124.5 | -41.34 | 12.29 | 101.17 |
| 2 | -8.89 | 23 | 121.2 | -25.82 | 12.29 | 101.27 |
| 3 | -10.19 | 24 | -146.29 | -37.49 | 12.29 | 101.43 |
| 4 | -11.19 | 24 | 121.2 | -25.82 | 11.061 | 101.27 |
| 5 | -12.89 | 26 | 121.2 | -25.82 | 9.832 | 101.27 |
| 6 | -7.69 | 70 | 162.64 | -10.77 | 12.29 | 101.32 |
| 7 | -9.89 | 71 | 162.64 | -10.77 | 11.061 | 101.32 |
| 8 | -11.59 | 72 | 162.64 | -10.77 | 9.832 | 101.32 |
| 9 | -15.09 | 72 | 68.09 | 36.69 | 12.29 | 101.79 |
| 10 | -14.79 | 87 | 77.21 | -53.01 | 12.29 | 101.02 |
| 11 | -18.39 | 90 | -81.01 | 41.32 | 12.29 | 101.86 |
| 12 | -18.79 | 103 | 61.1 | -74.26 | 12.29 | 100.88 |
| 13 | -12.79 | 135 | 78.53 | -47.01 | 12.29 | 101.67 |
| 14 | -14.49 | 144 | -97.95 | -52.88 | 12.29 | 101.71 |
| 15 | -16.39 | 238 | 89.55 | -62.14 | 12.29 | 100.84 |
| 16 | -20.89 | 298 | 47.78 | 55.99 | 12.29 | 101.71 |
| 17 | -21.59 | 468 | 9.05 | 53.71 | 12.29 | 100.99 |
| 18 | -21.59 | 505 | 25.66 | 46.93 | 12.29 | 100.85 |
| 19 | -23.49 | 603 | 11.27 | -77.39 | 12.29 | 100.78 |
| 20 | -24.79 | 616 | 30.51 | 49.14 | 12.29 | 101.9 |
| 21 | -23.69 | 693 | 27.14 | 49.86 | 12.29 | 100.84 |
| 22 | -23.39 | 729 | 23.11 | 58.53 | 12.29 | 100.83 |
| 23 | -29.29 | 774 | 4.53 | 66.94 | 12.29 | 101.79 |
| 24 | -30.49 | 951 | 50.16 | -91.67 | 12.29 | 101.99 |
| Ini. delay [ns] | XPR [dB] | PL [dB] | ZoA [°] | ASD [°] | ZSA [°] | ZSD [°] |
| 814 | 7 | 114.96 | 90 | 1.3 | 0 | 0.23 |
| UE speed [m/s] | UE DoT Az [°] | UE coordinates (x,y,z) [m] | BS coordinates (x,y,z) [m] | K-factor [dB] |  |  |
| 8.33 | 98.5 | (243.15,-17.48,1.5) | (0,0,10) | - |  |  |
| Way point | 6 |  |  |  |  |  |
| Cluster# | Power [dB] | Excess delay [ns] | AoA [°] | AoD [°] | ASA [°] | ZoD [°] |
| 1 | -18.8 | 0 | 96.49 | -63.08 | 7.52 | 98.77 |
| 2 | -5.4 | 42 | -156.53 | -2.95 | 7.52 | 99.25 |
| 3 | -7.6 | 44 | -156.53 | -2.95 | 6.768 | 99.25 |
| 4 | -11.4 | 51 | 113.84 | 29.7 | 7.52 | 99.57 |
| 5 | -13.6 | 59 | 113.84 | 29.7 | 6.768 | 99.57 |
| 6 | -15.9 | 63 | -53.04 | 40.52 | 7.52 | 99.88 |
| 7 | -9.4 | 64 | -156.53 | -2.95 | 6.016 | 99.25 |
| 8 | -15.3 | 74 | 113.84 | 29.7 | 6.016 | 99.57 |
| 9 | -12.9 | 84 | -80.54 | -29.75 | 7.52 | 98.83 |
| 10 | -21.3 | 169 | 125.91 | 53.28 | 7.52 | 98.51 |
| 11 | -12 | 209 | 96.9 | -30.2 | 7.52 | 99.62 |
| 12 | -17.8 | 239 | 79.55 | -54.4 | 7.52 | 99.89 |
| 13 | -22.1 | 244 | -131.29 | 45.12 | 7.52 | 100.12 |
| 14 | -20.6 | 274 | 113.84 | -60.97 | 7.52 | 99.96 |
| 15 | -16.2 | 276 | -56.61 | -46.41 | 7.52 | 98.74 |
| 16 | -16.7 | 336 | -67.58 | -48.52 | 7.52 | 98.66 |
| 17 | -18.1 | 448 | -84.18 | 55.7 | 7.52 | 98.7 |
| 18 | -21.6 | 490 | 137.02 | 49.83 | 7.52 | 100.01 |
| 19 | -23.7 | 502 | 160.41 | 44.4 | 7.52 | 98.34 |
| 20 | -24.3 | 527 | 167.81 | -42.51 | 7.52 | 98.4 |
| 21 | -22 | 550 | 131.26 | -54.78 | 7.52 | 100.12 |
| 22 | -25.3 | 583 | -155.91 | 42.25 | 7.52 | 98.47 |
| 23 | -35.1 | 1061 | 99.16 | -20.93 | 7.52 | 99.82 |
| Ini. delay [ns] | XPR [dB] | PL [dB] | ZoA [°] | ASD [°] | ZSA [°] | ZSD [°] |
| 803 | 10 | 114.76 | 90 | 1.73 | 0 | 0.03 |
| UE speed [m/s] | UE DoT Az [°] | UE coordinates (x,y,z) [m] | BS coordinates (x,y,z) [m] | K-factor [dB] |  |  |
| 0.833 | 81.43 | (240.63,-0.61,1.5) | (0,0,10) | - |  |  |
| Way point | 7 |  |  |  |  |  |
| Cluster# | Power [dB] | Excess delay [ns] | AoA [°] | AoD [°] | ASA [°] | ZoD [°] |
| 1 | -12.09 | 0 | -116.88 | -33.72 | 12.29 | 101.17 |
| 2 | -8.89 | 23 | 128.82 | -18.2 | 12.29 | 101.27 |
| 3 | -10.19 | 24 | -138.67 | -29.87 | 12.29 | 101.43 |
| 4 | -11.19 | 24 | 128.82 | -18.2 | 11.061 | 101.27 |
| 5 | -12.89 | 26 | 128.82 | -18.2 | 9.832 | 101.27 |
| 6 | -7.69 | 70 | 170.26 | -3.15 | 12.29 | 101.32 |
| 7 | -9.89 | 71 | 170.26 | -3.15 | 11.061 | 101.32 |
| 8 | -11.59 | 72 | 170.26 | -3.15 | 9.832 | 101.32 |
| 9 | -15.09 | 72 | 75.71 | 44.31 | 12.29 | 101.79 |
| 10 | -14.79 | 87 | 84.83 | -45.39 | 12.29 | 101.02 |
| 11 | -18.39 | 90 | -73.39 | 48.94 | 12.29 | 101.86 |
| 12 | -18.79 | 103 | 68.72 | -66.64 | 12.29 | 100.88 |
| 13 | -12.79 | 135 | 86.15 | -39.39 | 12.29 | 101.67 |
| 14 | -14.49 | 144 | -90.33 | -45.26 | 12.29 | 101.71 |
| 15 | -16.39 | 238 | 97.17 | -54.52 | 12.29 | 100.84 |
| 16 | -20.89 | 298 | 55.4 | 63.61 | 12.29 | 101.71 |
| 17 | -21.59 | 468 | 16.67 | 61.33 | 12.29 | 100.99 |
| 18 | -21.59 | 505 | 33.28 | 54.55 | 12.29 | 100.85 |
| 19 | -23.49 | 603 | 18.89 | -69.77 | 12.29 | 100.78 |
| 20 | -24.79 | 616 | 38.13 | 56.76 | 12.29 | 101.9 |
| 21 | -23.69 | 693 | 34.76 | 57.48 | 12.29 | 100.84 |
| 22 | -23.39 | 729 | 30.73 | 66.15 | 12.29 | 100.83 |
| 23 | -29.29 | 774 | 12.15 | 74.56 | 12.29 | 101.79 |
| 24 | -30.49 | 951 | 57.78 | -84.05 | 12.29 | 101.99 |
| Ini. delay [ns] | XPR [dB] | PL [dB] | ZoA [°] | ASD [°] | ZSA [°] | ZSD [°] |
| 812 | 7 | 114.94 | 90 | 1.3 | 0 | 0.23 |
| UE speed [m/s] | UE DoT Az [°] | UE coordinates (x,y,z) [m] | BS coordinates (x,y,z) [m] | K-factor [dB] |  |  |
| 8.33 | 54.59 | (242.97,14.91,1.5) | (0,0,10) | - |  |  |
| Way point | 8 |  |  |  |  |  |
| Cluster# | Power [dB] | Excess delay [ns] | AoA [°] | AoD [°] | ASA [°] | ZoD [°] |
| 1 | -18.8 | 0 | 122.01 | -37.56 | 7.52 | 980 |
| 2 | -5.4 | 42 | -131.01 | 22.57 | 7.52 | 98.48 |
| 3 | -7.6 | 44 | -131.01 | 22.57 | 6.768 | 98.48 |
| 4 | -11.4 | 51 | 139.36 | 55.22 | 7.52 | 98.8 |
| 5 | -13.6 | 59 | 139.36 | 55.22 | 6.768 | 98.8 |
| 6 | -15.9 | 63 | -27.52 | 66.04 | 7.52 | 99.11 |
| 7 | -9.4 | 64 | -131.01 | 22.57 | 6.016 | 98.48 |
| 8 | -15.3 | 74 | 139.36 | 55.22 | 6.016 | 98.8 |
| 9 | -12.9 | 84 | -55.02 | -4.23 | 7.52 | 98.06 |
| 10 | -21.3 | 169 | 151.43 | 78.8 | 7.52 | 97.74 |
| 11 | -12 | 209 | 122.42 | -4.68 | 7.52 | 98.85 |
| 12 | -17.8 | 239 | 105.07 | -28.88 | 7.52 | 99.12 |
| 13 | -22.1 | 244 | -105.77 | 70.64 | 7.52 | 99.35 |
| 14 | -20.6 | 274 | 139.36 | -35.45 | 7.52 | 99.19 |
| 15 | -16.2 | 276 | -31.09 | -20.89 | 7.52 | 97.97 |
| 16 | -16.7 | 336 | -42.06 | -23 | 7.52 | 97.89 |
| 17 | -18.1 | 448 | -58.66 | 81.22 | 7.52 | 97.93 |
| 18 | -21.6 | 490 | 162.54 | 75.35 | 7.52 | 99.24 |
| 19 | -23.7 | 502 | -174.07 | 69.92 | 7.52 | 97.57 |
| 20 | -24.3 | 527 | -166.67 | -16.99 | 7.52 | 97.63 |
| 21 | -22 | 550 | 156.78 | -29.26 | 7.52 | 99.35 |
| 22 | -25.3 | 583 | -130.39 | 67.77 | 7.52 | 97.7 |
| 23 | -35.1 | 1061 | 124.68 | 4.59 | 7.52 | 99.05 |
| Ini. delay [ns] | XPR [dB] | PL [dB] | ZoA [°] | ASD [°] | ZSA [°] | ZSD [°] |
| 1295 | 10 | 122.08 | 90 | 1.73 | 0 | 0.03 |
| UE speed [m/s] | UE DoT Az [°] | UE coordinates (x,y,z) [m] | BS coordinates (x,y,z) [m] | K-factor [dB] |  |  |
| 8.33 | -23.91 | (350.59,166.27,1.5) | (0,0,10) | - |  |  |
| Way point | 9 |  |  |  |  |  |
| Cluster# | Power [dB] | Excess delay [ns] | AoA [°] | AoD [°] | ASA [°] | ZoD [°] |
| LOS | -0.51 | 0 | -156.01 | 23.99 | 0 | 91.23 |
| 1 | -22.24 | 0 | -156.01 | 23.99 | 14.19 | 91.23 |
| 2 | -16.01 | 27 | -5.51 | 90.63 | 14.19 | 93.18 |
| 3 | -18.31 | 29 | -5.51 | 90.63 | 12.771 | 93.18 |
| 4 | -23.11 | 29 | 102.74 | 0.7 | 14.19 | 91.15 |
| 5 | -20.01 | 30 | -5.51 | 90.63 | 11.352 | 93.18 |
| 6 | -22.61 | 37 | 117.11 | 42.76 | 14.19 | 91.74 |
| 7 | -18.81 | 100 | -124.29 | 34.77 | 14.19 | 90.89 |
| 8 | -21.01 | 101 | -124.29 | 34.77 | 12.771 | 90.89 |
| 9 | -22.81 | 103 | -124.29 | 34.77 | 11.352 | 90.89 |
| 10 | -22.51 | 138 | -108.49 | 46.01 | 14.19 | 91.74 |
| 11 | -25.81 | 195 | -45.56 | 61.89 | 14.19 | 89.88 |
| 12 | -20.41 | 286 | 161.26 | 24.57 | 14.19 | 90.93 |
| 13 | -30.01 | 629 | 30.18 | 88.77 | 14.19 | 89.54 |
| 14 | -29.41 | 1081 | 43.39 | 90.74 | 14.19 | 93.34 |
| Ini. delay [ns] | XPR [dB] | PL [dB] | ZoA [°] | ASD [°] | ZSA [°] | ZSD [°] |
| 1322 | 8 | 102.91 | 90 | 5.79 | 0 | 1.27 |
| UE speed [m/s] | UE DoT Az [°] | UE coordinates (x,y,z) [m] | BS coordinates (x,y,z) [m] | K-factor [dB] |  |  |
| 8.33 | -66.13 | (362.14,161.15,1.5) | (0,0,10) | 9 |  |  |
| Way point | 10 |  |  |  |  |  |
| Cluster# | Power [dB] | Excess delay [ns] | AoA [°] | AoD [°] | ASA [°] | ZoD [°] |
| LOS | -0.51 | 0 | -178.48 | 1.52 | 0 | 91.14 |
| 1 | -22.24 | 0 | -178.48 | 1.52 | 14.19 | 91.14 |
| 2 | -16.01 | 27 | -27.98 | 68.16 | 14.19 | 93.09 |
| 3 | -18.31 | 29 | -27.98 | 68.16 | 12.771 | 93.09 |
| 4 | -23.11 | 29 | 80.27 | -21.77 | 14.19 | 91.06 |
| 5 | -20.01 | 30 | -27.98 | 68.16 | 11.352 | 93.09 |
| 6 | -22.61 | 37 | 94.64 | 20.29 | 14.19 | 91.65 |
| 7 | -18.81 | 100 | -146.76 | 12.3 | 14.19 | 90.8 |
| 8 | -21.01 | 101 | -146.76 | 12.3 | 12.771 | 90.8 |
| 9 | -22.81 | 103 | -146.76 | 12.3 | 11.352 | 90.8 |
| 10 | -22.51 | 138 | -130.96 | 23.54 | 14.19 | 91.65 |
| 11 | -25.81 | 195 | -68.03 | 39.42 | 14.19 | 89.79 |
| 12 | -20.41 | 286 | 138.79 | 2.1 | 14.19 | 90.84 |
| 13 | -30.01 | 629 | 7.71 | 66.3 | 14.19 | 89.45 |
| 14 | -29.41 | 1081 | 20.92 | 68.27 | 14.19 | 93.25 |
| Ini. delay [ns] | XPR [dB] | PL [dB] | ZoA [°] | ASD [°] | ZSA [°] | ZSD [°] |
| 1430 | 8 | 104.27 | 90 | 5.79 | 0 | 1.27 |
| UE speed [m/s] | UE DoT Az [°] | UE coordinates (x,y,z) [m] | BS coordinates (x,y,z) [m] | K-factor [dB] |  |  |
| 8.33 | -137.83 | (428.41,11.4,1.5) | (0,0,10) | 9 |  |  |

Annex B:
UE coordinate system

B.1	Reference coordinate system

There are three sets of coordinate systems used for FR1 MIMO OTA measurements. They are aligned with the following instructions to avoid misalignment and ambiguity.

The coordinate system for UE positioning is the guidance to specify the UE orientation in Annex A of [5]. The default position of DUT aligned to coordinate system is illustrated in Figure B.1-1. For convenience, name the coordinate axis of DUT coordinate system as xDUT, yDUT and zDUT.

Figure B.1-1: Illustration of DUT coordinate system

The coordinate system for channel model, which is used to specify channel models for FR1 MIMO OTA, is defined in Clause 7 of [7]. Similarly, the coordinate axes of channel model coordinate system are named as xCM, yCM and zCM. The channel model coordinate system is illustrated in Figure B.1-2.

Figure B.1-2: Illustration of channel model coordinate system

The coordinate system of anechoic chamber is also applied where the 16 measurement probes are evenly located in the azimuth plane. To avoid any chamber implementation flexibility/ambiguity, the x-axis of the chamber shall be aligned with one of the 16 probes. The OTA/channel model coordinate systems and probe placement are further clarified in Figure B.1-3. The x-axis of the coordinate system is aligned with the centre of a probe and the x, y, and z axes of the OTA coordinate system are aligned with the xCM, yCM, and zCM axes of the channel model coordinate system.

Figure B.1-3: Illustration of aligned OTA and channel model coordinate systems

B.2	DUT positioning guidelines

To assess the MIMO OTA performance for representative device orientations and orientations, four device orientations are tested, i.e., DMP, DML (both left and right tilt), and DMSU as defined in Clause A.3 of [5] and as illustrated in Figure B.2-1.

Figure B.2-1: Test Conditions: FS DMP (top left), FS DML-left tilt (top right), FS DML-right tilt (bottom left), FS DMSU (bottom right)

Four rotations, i.e., 0°, 90°, 180°, 270°, of the device (per orientation) within the test volume account for determining a statistical typical performance of the device in the specified spatial channel. Those 4 rotations are illustrated in Figure 6.3-2 for the DMP orientation.

Figure B.2-2: Four Rotations for the DMP Orientation: 0°, 90°, 180°, 270°

Annex C:
Measurement uncertainty

<Editor’s note: This Annex can be modified by RAN5 outcome. >

C.1	Preliminary MU assessment for MPAC

This clause defines the preliminary measurement uncertainty (MU) budget, as shown in Table C-1, for FR1 MPAC systems supporting Dynamic MIMO OTA test cases.

Table C-1: Preliminary measurement uncertainty budget for FR1 MPAC system and Dynamic MIMO OTA

| UID | Description of uncertainty contribution | Example value (410 MHz <f ≤ 3 GHz) [dB] | Example value (3 GHz < f ≤ 7.125GHz) [dB] | Distribution of the probability | Std Uncertainty (410 MHz < f ≤ 3 GHz) [dB] | Std Uncertainty (3 GHz < f ≤ 7.125GHz) [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |
| 1 | Mismatch for measurement process | 0 | 0 | U-Shaped | 0 | 0 |
| 2 | Measure distance uncertainty | 0 | 0 | Normal | 0 | 0 |
| 3 | Quality of quiet zone | 0.5 | 0.5 | Actual | 0.50 | 0.50 |
| 4 | Base Station simulator | 1.3 | 1.3 | Normal | 0.65 | 0.65 |
| 5 | Channel Emulator |  |  | Actual | 0.84 | 0.84 |
|  | - absolute output power | 1.5 | 1.5 | (Normal - power; |  |  |
|  | - output signal stability | 0.5 | 0.5 | rect-stability) |  |  |
|  | - output stability with temperature | 0.4 | 0.4 |  |  |  |
| 6 | Dynamic Channel Modelling (Note 1) | [0.75] | [0.75] | Normal | [0.38] | [0.38] |
| 7 | Amplifier uncertainties | 0.7 | 0.7 | Rectangular | 0.40 | 0.40 |
| 8 | Random uncertainty | 0.4 | 0.4 | Normal | 0.2 | 0.2 |
| 9 | Throughput measurement: output level step resolution | 0 | 0 | Rectangular | 0 | 0 |
| 10 | Signal flatness | 0 | 0 | Normal | 0 | 0 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |
| 11 | Mismatch for calibration process | 0.2 | 0.2 | U-Shaped | 0.14 | 0.14 |
|  | - loopback cable path |  |  |  |  |  |
|  | - system input path |  |  |  |  |  |
|  | - reference antenna |  |  |  |  |  |
| 12 | Reference antenna positioning misalignment | 0 | 0 | Normal | 0 | 0 |
| 13 | Quality of quiet zone | 0.5 | 0.5 | Rectangular | 0.29 | 0.29 |
| 14 | Total uncertainty of the Network Analyzer | 0.2 | 0.5 | Normal | 0.1 | 0.25 |
| 15 | Uncertainty of an absolute gain of the calibration antenna | 0.58 | 0.58 | Normal | 0.29 | 0.29 |
| 16 | Offset of the Phase Centre of the Reference Antenna | 0 | 0 | Normal | 0 | 0 |
| Total Expanded Uncertainty, U, with 95 % Confidence Interval |  |  |  |  | [2.71] | [2.75] |
| Note 1: This MU value will need to be addressed/finalized by RAN5. |  |  |  |  |  |  |

C.2	MU contribution descriptions for MPAC

Most MU elements are described further in Clause B.2 of TS 38.551 with the exception of the MU element ‘Dynamic Channel Modelling’ in Table C-1. This element takes into account the temporal variation of CDL models and the traversing of the UE on a synthesized route; its MU value of [0.75] dB was chosen arbitrarily for the preliminary RAN4 budget and needs to be further investigated and defined by RAN5.

Annex D:
Environmental requirements

D.1	Ambient temperature

All MIMO OTA test cases defined in this technical report should be performed at nominal temperature, i.e., room temperature (e.g. 18C - 28C).

D.2	Operating voltage

All test cases shall be performed in the normal voltage condition with the DUT operated in stand-alone battery powered mode. No extreme voltage testing is required. It is recommended to start the testing with a fully charged battery and conclude and/or pause testing before the battery has completely lost its charge.

Annex E:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-08 | RAN4#112 | R4-2411573 |  |  |  | Initial Skeleton | 0.0.1 |
| 2024-10 | RAN4#112bis | R4-2416433 |  |  |  | R4-2413536 TP on Dynamic MIMO Aspects R4-2416578 TP on Dynamic MIMO Aspects | 0.1.0 |
| 2025-02 | RAN4#114 | R4-2501749 |  |  |  | R4-2502176 TP for TR 38.762: DynMIMO CM Validation Details and Corrections R4-2501746 TP to 38.762 on terms and abbreviations | 0.2.0 |
| 2025-04 | RAN4#114bis | R4-2503991 |  |  |  | R4-2504729 TP for TR 38.762: Clarifications of Channel Models and Applicability | 0.3.0 |
| 2025-05 | RAN4#115 | R4-2506289 |  |  |  | R4-2506285 TP to 38.762 on General parts R4-2505845 TP to TR 38.762: DynMIMO CM Validation Details, Clarifications, and Corrections | 0.4.0 |
| 2025-08 | RAN4#116 | R4-2510816 |  |  |  | R4-2510814 TP to 38.762 on some General parts R4-2512587 TP to 38.762 on Annex D Environmental requirements R4-2512591 TP to TR 38.762: on performance metrics R4-2512685 TP to TR 38.762: MU and CM aspects | 0.5.0 |
| 2025-09 | RAN#109 |  |  |  |  | For one-step approval | 1.0.0 |

DoT affects Doppler shifts and consequently the temporal correlation of the emulated channel model. DUT orientation in the internal coordinate system of the channel model affects the PAS and consequently the mapping of clusters (multi-paths of the model) onto probes. Continuous and smooth change of them both prevents discontinuities in way-point instances that would cause instantaneous spreading of Doppler spectrum, immediate obsolescence of channel estimates, and possibly artificial bursts of bit errors in the received signal. An example of interpolation: Say DoT is +30° in the location of way-point a and -10° in way-point b, then even though the route from WPa to WPb is straight, the DoT and correspondingly the orientation of UE in the model continuously and linearly changes from +30° to -10° along the route between way-points.