| 3GPP TR 38.765 V0.2.0 (2025-12) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group RAN Access Network; Study on Integrated Sensing And Communication (ISAC) for NR (Release 20) |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	7

4	Performance metrics	7

4.1	Definitions of performance metrics	7

4.2	Performance objectives	8

5	Measurements	8

6	Performance evaluation	10

6.1	Evaluation methodologies	10

6.2	Evaluation Assumptions	10

6.3	Performance evaluation results	11

6.3.1	Baseline configuration 1	11

6.3.2	Baseline configuration 2	11

6.3.3	[Configuration x]	11

7	Network architecture	11

8	RAN-CN procedures and signalling	12

9	Conclusions	12

Annex <A>: Evaluation assumptions	13

Annex <B>: Evaluation results	16

Annex <X>: Change history		17

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

Current 5G-Advanced network design focuses primarily on data transmission, and although RAT-based positioning is supported, the specifications do not offer the in-built capability to detect objects that are not connected to the network. Integrated Sensing and Communication (ISAC) involves the simultaneous use of radio frequency (RF) signals for both sensing and communication purposes. If sensing capabilities are integrated into the design of the system, sensing may be offered as a service alongside communications. This integration can lead to improved spectrum efficiency, reduced latency, and enhanced reliability in various applications.

TR 22.837 identifies a very wide range of ISAC use cases, and requirements for such are defined in TS 22.137, in particular, detection and/or tracking of UAV (Uncrewed Aerial Vehicles). SA2 also has an ongoing Rel-20 study on Architecture Enhancement to support Integrated Sensing and Communication (FS_Sensing_ARC, SP-250401).

In Release 19, a channel modelling framework for ISAC to enable evaluation of sensing techniques for such use cases was added to the existing communication models in TR38.901. Both radar cross-section (RCS) and mobility of sensing targets and other objects in the environment was studied in the scope of channel modelling. However, there is a need for further study of ISAC techniques, including network architecture, procedures, signalling, and corresponding performance evaluations for UAV sensing target use cases.

This study item aims to study the following aspects for Integrated Sensing and Communication (ISAC):

Evaluate the performance of gNB-based mono-static sensing (i.e., single TRP with co-located sensing transmitter and receiver) for UAV use case [RAN1]

-	Identify and study metrics, measurements, and relevant measurement quantization for UAV use case

-	As baseline, existing DL NR waveform and DL NR reference signals are to be used for evaluations.

-	For other waveform and reference signals, companies are to share relevant information

-	No UE impacts

-	Deployment scenario and assumptions for channel model calibration for UAV sensing targets in the Rel-19 ISAC channel model SI [FS_Sensing_NR] are used as starting point for evaluation assumptions.

-	FR1 frequency range is prioritized.

Study the procedures, signaling between RAN and CN to support ISAC [RAN3]

Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3]

-	Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts.

-	No inter-gNB coordination will be studied.

-	Coordination with SA2 as necessary.

# 1	Scope

The present document is intended to capture the output of study item for "Study on Integrated Sensing And Communication (ISAC) for NR" [2]. CP-OFDM is considered as baseline waveform. The purpose of this TR is to document the following investigations for NR ISAC.

-	Performance evaluation of gNB-based mono-static sensing for UAV use case

-	Performance metrics and related performance objectives

-	Evaluation assumptions

-	Evaluation results

-	Measurements and quantization

-	Procedures and signalling between RAN and CN to support ISAC

-	Network architecture for gNB-based mono-static sensing for UAV use cases

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP RP-252819: "Revised SID: Study on Integrated Sensing And Communication (ISAC) for NR".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

CPI	Coherent Processing Interval

STX	Sensing Transmitter

SRX	Sensing Receiver

UAV	Uncrewed Aerial Vehicles

TRP	Transmission-Reception Point

LCS	Local coordinate system

GCS	Global coordinate system

# 4	Performance metrics

Editor’s note: This section is to capture the definition of performance metrics for the evaluation, and if agreed, the targeted KPI values

## 4.1	Definitions of performance metrics

In the evaluation of NR ISAC, the following performance metrics are considered:

-	Horizontal/vertical positioning accuracy is defined as the absolute value of the difference between the estimated horizontal/vertical position and the corresponding true position of a sensing target.

NOTE:	There should be only one estimated horizontal/vertical position corresponding to the true position of a sensing target.

-	Velocity accuracy is defined as the absolute value of the difference between the estimated velocity and the corresponding true velocity of a sensing target. For single TRP monostatic sensing, both the radial velocity accuracy and the 3D velocity accuracy can be estimated. The true radial velocity is the projection of true 3D velocity on the direction from TRP to target for TRP monostatic.

NOTE:	There should be only one estimated velocity corresponding to the true velocity of a sensing target.

-	Missed detection probability is defined as the conditional probability of not detecting the presence of a target when the target is actually present in the simulation area.

Pmd=n=0N-1DnMnN

where,

-	Dn is the number of missed targets in the drop n, i.e., the true target not associated with any detected object.

-	Mn is the number of true targets in the drop n.

-	N is total number of drops with at least one target per drop

-	False alarm probability Type 1 is defined for cases without true target dropped in simulation area. An object is detected when there is no target present in simulation area is considered a false alarm.

Pf1=n=0N-1QnN

where,

-	Qn equal to 1 if at least one object is detected when there is no target dropped in the simulation area in the drop n, otherwise Qn equal to 0.

-	N is the total number of drops without targets in the simulation area.

-	False alarm probability Type 2 is defined for cases with true target dropped in simulation area. An object is detected but not associated with any true target in the simulation area is considered as a false alarm.

Pf2=0≤n<NMn'≠0Dn'Mn'K

where,

-	Dn' is the number of detected objects but not associated with any true targets in the drop n.

-	Mn' is the total number of detected objects in the drop n.

-	K is number of drops with at least one detected object.

NOTE:	Both False Alarm Probability Types are mandatory.

Sensing resolution, sensing service latency and refreshing rate are not considered as performance metrics for the evaluation of NR ISAC.

For the purpose of performance metric calculation, association of the detected object(s) and the true target(s) should fulfil at least the following conditions:

-	One true target is associated with at most one detected object.

-	One detected object is associated with at most one true target.

-	The same association applies to miss detection, false alarm probability Type 2 and positioning/velocity accuracy.

Companies should report the method used for association of the detected object(s) and the true target(s).

## 4.2	Performance objectives

The following performance objectives are adopted for the evaluation of UAV use case with gNB-based monostatic sensing.

Table 4.2-1: Performance objectives

| Performance metrics | Values |
| --- | --- |
| Missed detection Probability | 5% |
| False Alarm Probability Type 1 | 5% |
| False Alarm Probability Type 2 | 5% |
| Horizontal Positioning Accuracy | 10 m with confidence level 90% |
| Vertical Positioning Accuracy | 10 m with confidence level 90% |
| Velocity Accuracy | 5 m/s with confidence level 90% |
| NOTE: Confidence level of the X% represents X percentile point of the cumulative distribution function (CDF) of the estimation errors |  |

# 5	Measurements

Editor’s note: This section is to include the definitions of measurement metrics, and measurement quantization except for the related evaluation results.

From physical layer perspective, the following measurements that may be reported from RAN are identified in the study of NR ISAC.

-	Level A: Raw data per Tx antenna port per OFDM symbol per RX antenna port per TRP for a given time stamp

-	Option A1: Amplitude and phase samples in time/delay domain of the estimated channel, i.e., Amplitude and phase values of channel impulse response

-	Option A2: Amplitude and phase per subcarrier in frequency domain of the estimated channel

-	Level B: Amplitude and phase profile of delay, and/or Doppler, and/or angle per TRP for a given time stamp by using window(s) of the [consecutive] samples in delay, Doppler and/or angle domain

-	Option B1: Delay-Doppler profile per Tx antenna port per Rx antenna port, which includes the amplitude and phase samples distributed across different delays and Doppler shifts.

-	Option B2: Delay-Angle profile per Tx antenna port per OFDM symbol, which includes the amplitude and phase samples distributed across different delays and spatial angles (e.g., Angle of Arrival).

-	Option B3: Delay-Doppler-Angle profile per Tx antenna port, which includes the amplitude and phase samples distributed across delay, Doppler, and angle domains.

-	Option B4: Delay profile per Tx antenna port per OFDM symbol per Rx antenna port, which includes the amplitude and phase samples distributed across different delays.

NOTE:	Level B is applicable for either LCS or GCS

-	Level C: per detected path/point measurements per Tx antenna port per TRP which may be reflected/scattered from scattering point(s).

-	Option C1: Delay/range, Doppler/velocity, one or multiple 3D angles, and [power/confidence metric] per detected path for a given time stamp. A path is associated with one couple {delay/range, Doppler/velocity, [power/confidence metric]}, and one or multiple 3D angles.

-	Option C2: Doppler/velocity, position, and [power/confidence metric] per detected path for a given time stamp. A path is associated with one doppler/velocity, [power/confidence metric], one or multiple positions

-	Option C3: Delay/range, Doppler/velocity, 3D angle, and [power/confidence metric] per detected path for a given time stamp. A path is associated with one delay, and one or multiple triple {Doppler/velocity, 3D angle, [power/confidence metric]}

-	Option C4: Delay/range, Doppler/velocity, 3D angle, and [power/confidence metric] per detected point for a given time stamp A point is associated with one range/delay, one Doppler/velocity, one 3D angle

-	Option C5: Position, velocity, and [power/confidence metric] per detected point for a given time stamp. A point is associated with one position, one velocity

NOTE:	Position can be defined in either LCS or GCS. Angle can be defined in either LCS or GCS. 3D angle refers to a pair of horizontal and vertical angles.

-	Level D: Object/target level measurement [per TRP or per gNB]. One or more value pair(s) {position, velocity} in GCS for a given time stamp is reported for a detected object/target

-	Option D1: Only one value pair {position, velocity} in GCS for a given time stamp is reported for a detected object/target. The association of multiple measurements across different time stamps for the same detected object/target is not reported.

-	Option D2: Only one value pair {position, velocity} in GCS for a given time stamp is reported for a detected object/target. The association of multiple measurements across different time stamps for the same detected object/target is reported.

-	Option D3: One or more value pairs {position, velocity} in GCS for a given time stamp are reported for a detected object/target. The association of multiple measurements across different time stamps for the same detected object/target is reported.

-	Option D4: One or more value pairs {position, velocity} in GCS for a given time stamp are reported for a detected object/target. The association of multiple measurements across different time stamps for the same detected object/target is not reported.

NOTE:	Velocity is 3D velocity for Option D2/D3.

NOTE:	For Level A/B/C, Tx antenna port means reference signal antenna port for sensing purpose.

# 6	Performance evaluation

Editor’s note: This section is to summarize the evaluation assumptions for UAV sensing per RAN1 agreements. Details in Annex A.

Editor’s note: This section is to summarize on evaluation results for UAV sensing per RAN1 agreements. Details in Annex B.

-	It includes the evaluation results based on NR waveform and DL NR reference signal

-	Depending on RAN1 discussions, it can also include other results based on other waveform and reference signals

-	It includes the evaluation results considering measurement quantization.

## 6.1	Evaluation methodologies

The following general procedure for performance evaluation of NR ISAC is provided.

1)	Simulation parameter configuration

2)	Sensing scenario generation, including the deployment of sensing Tx/Rx (STXs/SRXs)

3)	Dropping N target(s), where N is equal to 0 or larger than 0

4)	Channel generation and STX/SRX determination for the targets

5)	Sensing signal generation and passing the sensing signal to the generated channel

6)	Sensing signal processing at each SRX, optionally, sensing signal processing based on fusion from multiple STXs/SRXs

-	E.g., optionally, sensing signal processing based on tracking

7)	Sensing performance metric calculation.

## 6.2	Evaluation Assumptions

The evaluation assumptions for the evaluation of UAV use case with gNB-based monostatic sensing are provided in Annex A. In order to define the sensing resource ratio used in the evaluation of NR ISAC, three kinds of resources are defined

-	Type_1: Resources that are used for sensing signal transmission

-	Type_2: Part of Type_1 resources that are used for communication purpose

NOTE:	It is possible the Type_2 resource doesn’t exist.

-	Type_3: Resources that are not used for sensing signal transmission, and cannot be used for communication purpose due to sensing operation

Two options are provided to calculate the sensing resource ratio. Both options should be reported by companies.

-	Option 1: (Type_1 + Type_3) resources over all radio DL and UL resources

-	Option 2: (Type_1 - Type_2 + Type_3) resources over all radio DL and UL resources

NOTE:	If Type_2 resource doesn’t exist, two options are the same

In the evaluation on NR ISAC, company should report which sensing RS resources are considered as Type 2 resource and related reason.

Multiple configurations of the following parameters are defined to respectively analyse the evaluation results. Two baseline configurations are defined for evaluation purpose.

Table 6.1-1: Baseline configurations of evaluation assumptions

|  | Parameters | Baseline configuration 1 | Baseline configuration 2 |
| --- | --- | --- | --- |
| 1 | Scenario | (UMa-AV, 500m) | (UMa-AV, 500m) |
| 2 | Sensing Tx/Rx operating simultaneously or not | Sensing Tx/Rx operating simultaneously | Sensing Tx/Rx operating simultaneously |
| 3 | Carrier frequency | 4 or 4.9GHz | 4 or 4.9GHz |
| 4 | Max BS Tx power | 52 dBm | 37 dBm |
| 5 | BS antenna configuration | (M, N, P, Mg, Ng, Mp, Np) Tx: (8,8,2,1,1;4,8) Rx: (8,8,2,1,1;4,8) dH,dV = (0.5, 0.8)λ, +45°/-45° polarization | (M, N, P, Mg, Ng, Mp, Np) Tx: (8,8,2,1,1;4,8) Rx: (8,8,2,1,1;4,8) dH,dV = (0.5, 0.8)λ, +45°/-45° polarization |
| 6 | Number of targets per sector in the center site | N=5 | N=5 |
| 7 | Target vertical distribution | 25-300m | 25-300m |

  - Additional configurations can be defined for other assumptions of the parameters, based on reported evaluation results in RAN1 #124.

## 6.3	Performance evaluation results

### 6.3.1	Baseline configuration 1

[Editor’s note] this section is to capture observations on results for baseline configuration 1

### 6.3.2	Baseline configuration 2

[Editor’s note] this section is to capture observations on results for baseline configuration 2

### 6.3.3	[Configuration x]

[Editor’s note] this section is to capture observations on results for other configurations if agreed. This section may be split to one or more sections.

# 7	Network architecture

Editor’s note: This section is to capture the study outcome of network architecture. Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. No inter-gNB coordination will be studied.

# 8	RAN-CN procedures and signalling

Editor’s note: This section is to capture the study outcome of procedures and signaling aspects between RAN and CN for gNB-based monostatic sensing.

# 9	Conclusions

Editor’s note: TBA.

Annex <A>: Evaluation assumptions

Editor’s note: This annex is to include the agreed evaluation assumptions for UAV sensing per RAN1 agreements, which complements Clause 6.

-	It includes the evaluation assumptions based on NR waveform and DL NR reference signal

-	Depending on RAN1 discussions, it can also include other assumptions based on other waveforms and reference signals

In this clause, the evaluation assumptions for the evaluation of UAV use case with gNB-based monostatic sensing are provided. When sensing Tx/Rx operates simultaneously, the assumptions are summarized in Table A-1.

Table A-1: Evaluation assumptions

| Parameters |  | Assumptions |  |
| --- | --- | --- | --- |
|  |  | FR1 | FR2-1 (Optional) |
| Scenario |  | UMa-AV, optional RMa-AV | UMi-AV |
| Carrier frequency |  | 4 or 4.9 GHz Optional for FR1: 6 GHz | 30 GHz |
| System bandwidth |  | 100 MHz | 400 MHz |
| Numerology |  | SCS = 30 kHz | SCS = 120 kHz |
| BS Layout |  | Hexagonal grid, 7 macro sites, 3 sectors per site. 3 sectors with 30, 150, 270 degrees |  |
| Inter-BS (2D) distance |  | UMa-AV: 500 m, optional 1000 m RMa-AV: 1732 m | 200 m |
| Wrap-round |  | No wrap-round |  |
| BS antenna height |  | UMa-AV: 25 m RMa-AV: 35 m | 10 m |
| BS antenna configuration |  | (M, N, P, Mg, Ng; Mp, Np) for 4GHz, 4.9GHz - Configuration A - Tx: (8,8,2,1,1;4,8) - Rx: (8,8,2,1,1;4,8) - Configuration B as optional - Tx: (12,16,2,1,1;2,16) - Rx: (12,16,2,1,1;2,16) (M, N, P, Mg, Ng; Mp, Np) for 6GHz - Configuration A - Tx: (8,8,2,1,1;4,8) - Rx: (8,8,2,1,1;4,8) - Configuration B as optional - Tx: (16,16,2,1,1;4,16) - Rx: (16,16,2,1,1;4,16) dH,dV = (0.5, 0.8)λ, +45°/-45° polarization Optional: dH,dV = (0.5, 0.5)λ, +45°/-45° polarization | (M, N, P, Mg, Ng; Mp, Np) - Configuration A - Tx: (16,16,2,1,1;1,1) - Rx: (16,16,2,1,1;1,1) dH,dV = (0.5, 0.5 or 0.8)λ, +45°/-45° polarization |
| BS antenna radiation pattern |  | Table 9 in Report ITU-R M.2412 |  |
| BS antenna mechanic tilt |  | 90° in GCS (pointing to horizontal direction) |  |
| BS antenna electrical tilt |  | Option 1: no electrical tilt Option 2: 102° in GCS |  |
| Polarized antenna model |  | Model-2 in clause 7.3.2 in TR 38.901 |  |
| Antenna isolation |  | 65 dB, 80 dB | 80 ~ 100 dB |
| Max BS Tx power |  | 37 dBm, 52 dBm. See note 1 | 30 dBm |
| Inter-site interference |  | Not modelled |  |
| Self-interference |  | The residual leakage interference/noise is modelled e.g. by additional additive white Gaussian noise, -94+X dBm in 100 MHz, X is up to company report. Companies to provide details on their modelling. See note 2 |  |
| Co-site inter-sector interference |  | Not modelled |  |
| Adjacent channel interference |  | Not modelled |  |
| BS receiver noise figure |  | 5 dB | 7 dB |
| Sensing target | Target type | UAV with small size (0.3m x 0.4m x 0.2m) |  |
|  | 3D distribution | N targets per sector in the center site only - N = 5 - Optional: N is uniformly distributed from 1 to 10 Horizontal plane: Uniformly distributed in a sector Vertical plane: Uniformly distributed between 25m and 300m, optionally between 1.5m and 300m. |  |
|  | Mobility | Horizontal speed: uniformly distributed between 0 and 180km/h Vertical speed: 0km/h |  |
|  | Minimum BS-target 3D distance | 10 m |  |
|  | Minimum target-target (3D) distance | 10 m |  |
|  | Outdoor/indoor proportion | 100% outdoor |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | Orientation | Random in horizontal domain |  |
|  | RCS model | RCS model 1 for UAV with small size |  |
| gNB-target link |  | TRP-UAV link in Table 7.9.3-2 in TR 38.901, using Clause B.1.3 in TR 36.777 |  |
| Concatenation of TX-target and target-RX links |  | Up to company choice between two options for concatenation defined in Step 9 in clause 7.9.4.1 in TR 38.901 |  |
| The power threshold for path dropping after concatenation for target channel |  | -25 dB and -40 dB are respectively used for the two options for concatenation |  |
| NOTE 1: The above options are calculated with BS_maxpower = BS Rx saturation power + antenna isolation by assuming the BS Rx saturation power = -28dBm and the antenna isolation = 65dB and 80dB, respectively. NOTE 2: X = -Infinity corresponds to not modelling self-interference. |  |  |  |

Besides the evaluation parameters provided in Table A-1, the following assumptions are up to company report:

-	To model self-interference, value of X to derive the power of the additional additive white Gaussian noise to model the residual leakage interference/noise.

-	Length of Coherent Processing Interval (CPI).

-	Tx beam information (number of Tx beams, wide/narrow Tx beam) being used at TRP.

-	RE mapping of sensing RS, and assumed TDD UL/DL configuration if applicable.

-	Sensing resource ratio.

-	High-level sensing signal/data processing method, e.g., 2D FFT, MUSIC, and any other methods.

-	Optionally, the maximum BS Tx power when it is assumed that Tx and Rx don’t operate simultaneously. Companies should report how the maximum BS Tx power is derived.

-	Sensing signal processing and ISAC channel generation.

-	Whether a same target is modelled in the ISAC channel of single, multiple or all STXs/SRXs?

-	Company should report how to determine the single or multiple STXs/SRXs for a target.

-	If the evaluation results are derived by measurement reports from multiple/all STXs/SRXs, companies should report how measurement reports from multiple/all STXs/SRXs are used.

-	Beam set at TRxP for FR2-1.

-	Additional configuration information for FR2-1.

-	How target trajectory is modeled if evaluation results on UAV tracking is reported.

Annex <B>: Evaluation results

Editor’s note: This annex is to include the detailed evaluation results for UAV sensing per RAN1 agreements, which complements Clause 6.

Annex <X>: Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-10 | RAN1 #122-bis | R1-2507422 |  |  |  | TR Skeleton | 0.0.1 |
| 2025-11 | RAN1 #123 | R1-2509522 |  |  |  | RAN1 agreements till RAN1 #122bis | 0.1.1 |
| 2025-12 | RAN1 #123 | R1-2509632 |  |  |  | RAN1 agreements in RAN1 #123 | 0.2.0 |