#### 7.1.1.5	Discontinuous reception

##### 7.1.1.5.0	DRX Common Definitions

FirstSlot is the First DL Slot in the subframe, which is 0 for both FDD and TDD as per default configuration in 38.5081-1[4] TDD-UL-DL-Config Table 4.6.3-192

LastDLSlot is the Last DL Slot in a frame; for FDD (except for HD-FDD) numerology =0 it is slot 9, numerology=1 it is slot 19, numerology=2 it is slot 39. For TDD as per default configuration in 38.5081-1[4] TDD-UL-DL-Config Table 4.6.3-192, for numerology =0, it is slot 7, numerology=1 it is slot 16, numerology=3 it is slot 77. For HD-FDD as per default configuration in TS 38.508-1[4] Table 4.6.3-162, for numerology=0, it is slot 8.

LastULSlot is the Last UL Slot in a frame; for FDD/TDD numerology =0 it is slot 9, numerology=1 it is slot 18(Second Last as 2 Consecutive UL Slots), numerology=3 it is slot 79; the PDCCH for UL grant is sent K2= 4 Slot earlier.

##### 7.1.1.5.1	DRX operation / Short cycle not configured / Parameters configured by RRC

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { Long DRX cycle is configured and [(SFN * 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset }

then { UE starts the OnDurationTimer and monitors the PDCCH for OnDurationTimer PDCCH-Occasions }

}

(2)

with { UE in RRC_CONNECTED state }

ensure that {

when { Long DRX cycle is configured and a new DL transmission is indicated on the PDCCH during Active Time }

then { UE starts or restarts the Drx-InactivityTimer and monitors the PDCCH for Drx-InactivityTimer PDCCH occasions starting from the next PDCCH occasion of the PDCCH occasion where the DL new transmission was indicated }

}

(3)

with { UE in  RRC_CONNECTED state }

ensure that {

when { Long DRX cycle is configured and if a HARQ RTT Timer expires in this PDCCH Occasion and the data in the soft buffer of the corresponding HARQ process was not successfully decoded }

then { UE starts the drx-RetransmissionTimer-DL for the corresponding HARQ process and monitors the PDCCH for drx-RetransmissionTimer consecutive PDCCH Occasion }

}

(4)

with { UE in RRC_CONNECTED state }

ensure that {

when { Long DRX cycle is configured and an uplink grant for a pending HARQ retransmission can occur in this PDCCH occasion }

then { UE monitors the PDCCH in this PDCCH occasion }

}

7.1.1.5.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.321, clause 5.7. Unless otherwise stated these are Rel-15 requirements.

[TS 38.321, clause 5.7]

The MAC entity may be configured by RRC with a DRX functionality that controls the UE’s PDCCH monitoring activity for the MAC entity's C-RNTI, CS-RNTI, INT-RNTI, SFI-RNTI, SP-CSI-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, and TPC-SRS-RNTI. When using DRX operation, the MAC entity shall also monitor PDCCH according to requirements found in other subclauses of this specification. When in RRC_CONNECTED, if DRX is configured, the MAC entity may monitor the PDCCH discontinuously using the DRX operation specified in this subclause; otherwise the MAC entity shall monitor the PDCCH continuously.

RRC controls DRX operation by configuring the following timers:

-	drx-onDurationTimer: the duration at the beginning of a DRX Cycle;

-	drx-SlotOffset: the delay before starting the drx-onDurationTimer;

-	drx-InactivityTimer: the duration after the PDCCH occasion in which a PDCCH indicates an new UL or DL transmission for the MAC entity;

-	drx-RetransmissionTimerDL (per DL HARQ process): the maximum duration until a DL retransmission is received;

-	drx-RetransmissionTimerUL (per UL HARQ process): the maximum duration until a grant for UL retransmission is received;

-	drx-LongCycle StartOffset: the Long DRX cycle and drx-StartOffset which defines the subframe where the Long and Short DRX Cycle starts;

-	drx-ShortCycle (optional): the Short DRX cycle;

-	drx-ShortCycleTimer (optional): the duration the UE shall follow the Short DRX cycle;

-	drx-HARQ-RTT-TimerDL (per DL HARQ process): the minimum duration before a DL assignment for HARQ retransmission is expected by the MAC entity;

-	drx-HARQ-RTT-TimerUL (per UL HARQ process): the minimum duration before a UL HARQ retransmission grant is expected by the MAC entity.

When a DRX cycle is configured, the Active Time includes the time while:

-	drx-onDurationTimer or drx-InactivityTimer or drx-RetransmissionTimerDL or drx-RetransmissionTimerUL or ra-ContentionResolutionTimer (as described in subclause 5.1.5) is running; or

-	a Scheduling Request is sent on PUCCH and is pending (as described in subclause 5.4.4); or

-	a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the random access preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in subclause 5.1.4).

When DRX is configured, the MAC entity shall:

1>	if a MAC PDU is received in a configured downlink assignment:

2>	start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback;

2>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process.

1>	if a MAC PDU is transmitted in a configured uplink grant:

2>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first repetition of the corresponding PUSCH transmission;

2>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process.

1>	if a drx-HARQ-RTT-TimerDL expires:

2>	if the data of the corresponding HARQ process was not successfully decoded:

3>	start the drx-RetransmissionTimerDL for the corresponding HARQ process.

1>	if an drx-HARQ-RTT-TimerUL expires:

2>	start the drx-RetransmissionTimerUL for the corresponding HARQ process.

1>	if a DRX Command MAC CE or a Long DRX Command MAC CE is received:

2>	stop drx-onDurationTimer;

2>	stop drx-InactivityTimer.

1>	if drx-InactivityTimer expires or a DRX Command MAC CE is received:

2>	if the Short DRX cycle is configured:

3>	start or restart drx-ShortCycleTimer in the first symbol after the expiry of drx-HARQ-RTT-TimerDL.;

3>	use the Short DRX Cycle.

2>	else:

3>	use the Long DRX cycle.

1>	if drx-ShortCycleTimer expires:

2>	use the Long DRX cycle.

1>	if a Long DRX Command MAC CE is received:

2>	stop drx-ShortCycleTimer;

2>	use the Long DRX cycle.

1>	if the Short DRX Cycle is used, and [(SFN x 10) + subframe number] modulo (drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle); or

1>	if the Long DRX Cycle is used, and [(SFN x 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset:

2>	if drx-SlotOffset is configured:

3>	start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe.

2>	else:

3>	start drx-onDurationTimer.

1>	if the MAC entity is in Active Time:

2>	monitor the PDCCH;

2>	if the PDCCH indicates a DL transmission or if a DL assignment has been configured:

3>	start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process immediately after the corresponding PUCCH transmission;

3>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process.

2>	if the PDCCH indicates a UL transmission or if a UL grant has been configured:

3>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process immediately after the first repetition of the corresponding PUSCH transmission;

3>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process.

2>	if the PDCCH indicates a new transmission (DL or UL):

3>	start or restart drx-InactivityTimer.

1>	else (i.e. not part of the Active Time):

2>	not report CQI/PMI/RI on PUCCH.

7.1.1.5.1.3	Test description

7.1.1.5.1.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that set to return no data in uplink.

7.1.1.5.1.3.2	Test procedure sequence

Table 7.1.1.5.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits RRCReconfiguration to configure specific DRX parameters. (Note 6) | <-- | - | - | - |
| 2 | The UE transmits RRCReconfigurationComplete. (Note 7) | --> | - | - | - |
| 3 | In the first PDCCH occasion when the Drx-onDurationTimer is running, the SS indicates the transmission of a DL MAC PDU on the PDCCH. | <-- | MAC PDU | - | - |
| 4 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 3? | --> | HARQ ACK | 1 | P |
| 5 | At least drx-InactivityTimer PDCCH occasions after the transmission of the MAC PDU in Step 3 has been indicated (This means the next DRX cycle or later after Step 2) in the last PDCCH occasion while the drx-onDurationTimer is still running, the SS indicates the transmission a DL MAC PDU on the PDDCH. (Note 4). | <-- | MAC PDU | - | - |
| 6 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 5? | --> | HARQ ACK | 1 | P |
| 7 | drx-InactivityTimer PDCCH-occasions after the transmission of the MAC PDU transmitted in step 5 was indicated on the PDCCH, the SS indicates the transmission of a DL MAC PDU on the PDCCH. (Note 4) | <-- | MAC PDU | - | - |
| 8 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 7? | --> | HARQ ACK | 2 | P |
| 9 | At least drx-InactivityTimer PDCCH occasions after the transmission of the MAC PDU in Step 7 has been indicated (This means the next DRX cycle or later after Step 5) and in the last PDCCH occasion before the Drx-onDurationTimer expires, the SS indicates the transmission of a DL MAC PDU on the PDDCH. The DL MAC PDU transmitted is invalid. (Note 1, Note 4) | <-- | Invalid MAC PDU | - | - |
| 10 | Check: Does the UE transmit a HARQ NACK for the DL MAC PDU in Step 9? | --> | HARQ NACK | 1 | P |
| 11 | In the first PDCCH occasion when the Drx-RetransmissionTimerDL for the MAC PDU in Step 9 is started (i.e. after expiry of drx-HARQ-RTT-TimerDL after step 9), the SS indicates the transmission of a DL MAC PDU on the PDCCH. | <-- | MAC PDU | - | - |
| 12 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 11? | --> | HARQ ACK | 3 | P |
| 13 | At least drx-InactivityTimer PDCCH occasions after the transmission of the DL MAC PDU in Step 11 has been indicated (This means the next DRX cycle or later after Step 11) and last PDCCH occasion before the Drx-onDurationTimer expires, the SS indicates the transmission of DL MAC PDU on the PDCCH. The DL MAC PDU transmitted is invalid. (Note 1, Note 4) | <-- | Invalid MAC PDU | - | - |
| 14 | Check: Does the UE transmit a HARQ NACK for the DL MAC PDU in Step 13? | --> | HARQ NACK | 1 | P |
| 15 | In the last PDCCH occasion when the drx-RetransmissionTimerDL for MAC PDU in Step 13 is still running, the SS indicates the transmission of a DL MAC PDU on the PDCCH. | <-- | MAC PDU | - | - |
| 16 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 15? | --> | HARQ ACK | 3 | P |
| 17 | The SS is configured for Uplink Grant Allocation Type [0]. At least drx-InactivityTimer PDCCH subframes after the transmission of the DL MAC PDU in Step 15 has been indicated in the last PDCCH occasion when the onDuratiopnTimer is still running (This means the next DRX cycle or later after Step 9), the SS indicates an UL grant to the UE on the PDCCH. (Note 4) | <-- | UL grant on PDCCH | - | - |
| 18 | Check: Does the UE transmit a Buffer Status Report on the UL indicating an empty buffer? | --> | Buffer Status Report MAC control element | 1 | P |
| 19 | In the last PDCCH occasion when the drx-RetransmissionTimer-UL for MAC PDU from Step 17 is still running, the SS indicates the transmission of a DL MAC PDU on the PDCCH. | <-- | MAC PDU | - | - |
| 20 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 19? | --> | HARQ ACK | 4 | P |
| Note 1: Invalid MAC PDU is a MAC PDU that fails the CRC check. Note 2: All the DL MAC PDU are transmitted with the NDI set on the PDCCH. Note 3: Timer tolerances for the MAC DRX related timers measured in PDCCH occasions is 0. These timers are: drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL and drx-HARQ-RTT-TimerUL. Note 4: The drx-InactivityTimer is started in the next PDCCH occasion of the PDCCH occasion where DL new transmission is indicated. Note 5: The timer values expressed in number of slots. Note 6: For EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration 36.508 [7], Table 4.6.1-8 using condition EN-DC_EmbedNR_RRCRecon. Note 7: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. |  |  |  |  |  |

7.1.1.5.1.3.3	Specific message contents

Table 7.1.1.5.1.3.3-1: RRCReconfiguration (step 1, Table 7.1.1.5.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig |  | NR |
| secondaryCellGroup | CellGroupConfig |  | EN-DC |
| nonCriticalExtension SEQUENCE { |  |  | NR |
| masterCellGroup | CellGroupConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.5.1.3.3-2: CellGroupConfig (Table 7.1.1.5.1.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| mac-CellGroupConfig SEQUENCE { |  |  |  |
| drx-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| drx-onDurationTimer | ms20 |  |  |
| drx-InactivityTimer | ms10 |  |  |
| drx-HARQ-RTT-TimerDL | 56 | Number of slots=4 due to number of symbol per slot=14 | =0,1,2,3,4 ( 2 with normal CP) |
|  | 48 | Number of slots=4 due to number of symbol per slot=12 | = 2 with external CP |
| drx-HARQ-RTT-TimerUL | 56 | Number of slots=4 due to number of symbol per slot=14 | =0,1,2,3,4 ( 2 with normal CP) |
|  | 48 | Number of slots=4 due to number of symbol per slot=12 | = 2 with external CP |
| drx-RetransmissionTimerDL | sl8 |  |  |
| drx-RetransmissionTimerUL | sl8 |  |  |
| drx-LongCycleStartOffset CHOICE { |  |  |  |
| ms640 | 7 |  |  |
| } |  |  |  |
| shortDRX | Not present |  |  |
| drx-SlotOffset | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 7.1.1.5.2	DRX operation / Short cycle not configured / Long DRX command MAC control element reception

7.1.1.5.2.1	Test Purpose (TP)

(1)

with { UE in CONNECTED mode }

ensure that {

when { long DRX cycle is configured and a DRX Command MAC control element is received }

then { UE successfully decodes the MAC control PDU }

}

(2)

with { UE in CONNECTED mode }

ensure that {

when { long DRX cycle is configured and the HARQ RTT Timer is running and a DRX Command MAC control element is received }

then { UE continues running the HARQ RTT timer }

}

(3)

with { UE in CONNECTED mode }

ensure that {

when { long DRX cycle is configured and the drx-RetransmissionTimer is running and a DRX Command MAC control element is received }

then { UE continues running the drx-RetransmissionTimer and monitors the PDCCH }

}

7.1.1.5.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.321, clause 5.7. Unless otherwise stated these are Rel-15 requirements.

[TS 38.321, clause 5.7]

The MAC entity may be configured by RRC with a DRX functionality that controls the UE’s PDCCH monitoring.  Activity for the MAC entity's C-RNTI, CS-RNTI, INT-RNTI, SFI-RNTI, SP-CSI-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, and TPC-SRS-RNTI. When using DRX operation, the MAC entity shall also monitor PDCCH according to requirements found in other subclauses of this specification. When in RRC_CONNECTED, if DRX is configured, the MAC entity may monitor the PDCCH discontinuously using the DRX operation specified in this subclause; otherwise the MAC entity shall monitor the PDCCH continuously.

RRC controls DRX operation by configuring the following timers:

-	drx-onDurationTimer: the duration at the beginning of a DRX Cycle;

-	drx-SlotOffset: the delay  before starting the drx-onDurationTimer;

-	drx-InactivityTimer: the duration after the PDCCH occasion in which a PDCCH indicates a new UL or DL transmission for the MAC entity;

-	drx-RetransmissionTimerDL (per DL HARQ process): the maximum duration until a DL retransmission is received;

-	drx-RetransmissionTimerUL (per UL HARQ process): the maximum duration until a grant for UL retransmission is received;

-	drx-LongCycle StartOffset: the Long DRX cycle and drx-StartOffset which defines the subframe where the Long and Short DRX Cycle starts;

-	drx-ShortCycle (optional): the Short DRX cycle;

-	drx-ShortCycleTimer (optional): the duration the UE shall follow the Short DRX cycle;

-	drx-HARQ-RTT-TimerDL (per DL HARQ process): the minimum duration before a DL assignment for HARQ retransmission is expected by the MAC entity;

-	drx-HARQ-RTT-TimerUL (per UL HARQ process): the minimum duration before a UL HARQ retransmission grant is expected by the MAC entity.

When a DRX cycle is configured, the Active Time includes the time while:

-	drx-onDurationTimer or drx-InactivityTimer or drx-RetransmissionTimerDL or drx-RetransmissionTimerUL or ra-ContentionResolutionTimer (as described in subclause 5.1.5) is running; or

-	a Scheduling Request is sent on PUCCH and is pending (as described in subclause 5.4.4); or

-	a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the random access preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in subclause 5.1.4).

When DRX is configured, the MAC entity shall:

1>	if a MAC PDU is received in a configured downlink assignment:

2>	start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback;

2>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process.

1>	if a MAC PDU is transmitted in a configured uplink grant:

2>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first repetition of the corresponding PUSCH transmission;

2>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process.

1>	if a drx-HARQ-RTT-TimerDL expires:

2>	if the data of the corresponding HARQ process was not successfully decoded:

3>	start the drx-RetransmissionTimerDL for the corresponding HARQ process.

1>	if an drx-HARQ-RTT-TimerUL expires:

2>	start the drx-RetransmissionTimerUL for the corresponding HARQ process.

1>	if a DRX Command MAC CE or a Long DRX Command MAC CE is received:

2>	stop drx-onDurationTimer;

2>	stop drx-InactivityTimer.

1>	if drx-InactivityTimer expires or a DRX Command MAC CE is received:

2>	if the Short DRX cycle is configured:

3>	start or restart drx-ShortCycleTimer in the first symbol after the expiry of drx-HARQ-RTT-TimerDL.;

3>	use the Short DRX Cycle.

2>	else:

3>	use the Long DRX cycle.

1>	if drx-ShortCycleTimer expires:

2>	use the Long DRX cycle.

1>	if a Long DRX Command MAC CE is received:

2>	stop drx-ShortCycleTimer;

2>	use the Long DRX cycle.

1>	if the Short DRX Cycle is used, and [(SFN x 10) + subframe number] modulo (drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle); or

1>	if the Long DRX Cycle is used, and [(SFN x 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset:

2>	if drx-SlotOffset is configured:

3>	start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe.

2>	else:

3>	start drx-onDurationTimer.

1>	if the MAC entity is in Active Time:

2>	monitor the PDCCH;

2>	if the PDCCH indicates a DL transmission or if a DL assignment has been configured:

3>	start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process immediately after the corresponding PUCCH transmission;

3>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process.

2>	if the PDCCH indicates a UL transmission or if a UL grant has been configured:

3>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process immediately after the first repetition of the corresponding PUSCH transmission;

3>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process.

2>	if the PDCCH indicates a new transmission (DL or UL):

3>	start or restart drx-InactivityTimer.

1>	else (i.e. not part of the Active Time):

2>	not report CQI/PMI/RI on PUCCH.

7.1.1.5.2.3	Test description

7.1.1.5.2.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that set to return no data in uplink.

7.1.1.5.2.3.2	Test procedure sequence

For FDD, NormalSLT(current SFN,current sub-frame, current slot,y)=y; For TDD, NormalSLT(current SFN, current slot,y) counts the minimum number of normal slots needed to cover y number of PDCCH-occasions(slots) until next PDCCH-occasion(slot) available, starting from current slot on current SFN.

Table 7.1.1.5.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits RRCReconfiguration to configure specific DRX parameters. (Note 5) | <-- | - | - | - |
| 2 | The UE transmits RRCReconfigurationComplete. (Note 6) | --> | - | - | - |
| 3 | In a PDCCH occasion which is X PDCCH sub frames before the PDCCH occasion in which the onDurationTimer expires, with X < drx-onDurationTimer, the SS indicates the transmission of a DL MAC PDU on the PDCCH. The SS transmits an MAC PDU. | <-- | MAC PDU | - | - |
| 4 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 3? | --> | HARQ ACK | 1 | P |
| 5 | In a PDCCH occasion before the drx-onDurationTimer expires, the SS indicates the transmission of a DL MAC PDU on the PDCCH. The SS transmits a DL MAC PDU with DRX MAC Control element. UE successfully decodes the MAC PDU and starts the long DRX cycle. | <-- | MAC PDU(DRX MAC Control element) | - | - |
| 6 | Check: Does the UE transmit a HARQ ACK? | --> | HARQ ACK | 1 | P |
| 6A | In a PDCCH occasion before the Long DRX cycle ends, the SS indicates the transmission of a DL MAC PDU on the PDCCH. The SS transmits a DL MAC PDU | <-- | MAC PDU |  |  |
| 6B | Check: Does the UE transmit a HARQ ACK? | --> | HARQ ACK | 1 | F |
| 7 | On the next or later DRX cycle than the one used for Step 3 and on a PDCCH occasion which is X PDCCH sub frames before the PDCCH occasion in which the onDurationTimer expires, with X < drx-onDurationTimer,the SS indicates the transmission of a DL MAC PDU. The SS transmits an invalid MAC PDU. (Note 1) | <-- | MAC PDU | - | - |
| 8 | Check: Does the UE transmit a HARQ NACK? | --> | HARQ NACK | - | P |
| 8A | In a PDCCH occasion before the Drx-HARQ-RTT-TimerDL for the MAC PDU indicated in Step 7 expires, the SS indicates the transmission of a DL MAC PDU on the PDCCH. The SS transmits a DL MAC PDU with DRX MAC Control element. | <-- | MAC PDU(DRX MAC Control element) | - | - |
| 8B | Check: Does the UE transmit a HARQ ACK? | --> | HARQ ACK | 2,3 | P |
| 9 | In a PDCCH occasion when the drx-RetransmissionTimer for the MAC PDU indicated in Step 7 is still running,, the SS indicates the transmission of a DL MAC PDU. The SS transmits a DL MAC PDU with DRX MAC Control element. | <-- | MAC PDU(DRX MAC Control element) | - | - |
| 10 | Check: Does the UE transmit a HARQ ACK? | --> | HARQ ACK | 2,3 | P |
| 11 | In the last sub frame when the Drx-RetransmissionTimer for the DL MAC PDU indicated on the PDCCH in Step 7 is still running, the SS indicates the transmission of a DL MAC PDU. | <-- | MAC PDU | - | - |
| 12 | Check: Does the UE transmit a HARQ ACK? | --> | HARQ ACK | 2,3 | P |
| Note 1: Invalid MAC PDU is a MAC PDU that fails the CRC check. Note 2: All DL MAC PDUs are transmitted with the NDI set on the PDCCH. Note 3: Timer tolerances for the MAC DRX related timers measured in PDCCH occasions(slots). These timers are: drx-InactivityTimer, drx-RetransmissionTimer, Drx-HARQ-RTT-TimerDL. Note 5: For EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration 36.508 [7], Table 4.6.1-8 using condition EN-DC_EmbedNR_RRCRecon. Note 6: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete |  |  |  |  |  |

7.1.1.5.2.3.3	Specific message contents

Table 7.1.1.5.2.3.3-1: RRCReconfiguration (step 1, Table 7.1.1.5.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig |  | NR |
| secondaryCellGroup | CellGroupConfig |  | EN-DC |
| nonCriticalExtension SEQUENCE { |  |  | NR |
| masterCellGroup | CellGroupConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.5.2.3.3-2: CellGroupConfig (Table 7.1.1.5.2.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| mac-CellGroupConfig SEQUENCE { |  |  |  |
| drx-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| drx-onDurationTimer | ms40 |  |  |
| drx-InactivityTimer | ms10 |  |  |
| drx-HARQ-RTT-TimerDL | 56 | Number of slots=4 due to number of symbol per slot=14 | =0,1,2,3,4 ( 2 with normal CP) |
|  | 48 | Number of slots=4 due to number of symbol per slot=12 | = 2 with external CP |
| drx-HARQ-RTT-TimerUL | 56 | Number of slots=4 due to number of symbol per slot=14 | =0,1,2,3,4 ( 2 with normal CP) |
|  | 48 | Number of slots=4 due to number of symbol per slot=12 | = 2 with external CP |
| drx-RetransmissionTimerDL | sl80 |  |  |
| drx-RetransmissionTimerUL | sl80 |  |  |
| drx-LongCycleStartOffset CHOICE { |  |  |  |
| ms640 | 7 |  |  |
| } |  |  |  |
| shortDRX | Not present |  |  |
| drx-SlotOffset | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 7.1.1.5.3	DRX operation / Short cycle configured / Parameters configured by RRC

7.1.1.5.3.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { Short DRX cycle and drx-SlotOffset is configured and [(SFN * 10) + subframe number] modulo drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle) }

then { UE starts the OnDurationTimer after drx-SlotOffset and monitors the PDCCH for OnDurationTimer PDCCH-subframes }

}

(2)

with { UE in RRC_CONNECTED state }

ensure that {

when { drxShortCycleTimer is expired and [(SFN * 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset }

then { UE starts the OnDurationTimer after drx-SlotOffset and monitors the PDCCH for OnDurationTimer PDCCH-subframes }

}

7.1.1.5.3.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.321, clause 5.7. Unless otherwise stated these are Rel-15 requirements.

[TS 38.321, clause 5.7]

The MAC entity may be configured by RRC with a DRX functionality that controls the UE's PDCCH monitoring activity for the MAC entity's C-RNTI, CS-RNTI, INT-RNTI, SFI-RNTI, SP-CSI-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, and TPC-SRS-RNTI. When using DRX operation, the MAC entity shall also monitor PDCCH according to requirements found in other clauses of this specification. When in RRC_CONNECTED, if DRX is configured, for all the activated Serving Cells, the MAC entity may monitor the PDCCH discontinuously using the DRX operation specified in this clause; otherwise the MAC entity shall monitor the PDCCH as specified in TS 38.213 [6].

RRC controls DRX operation by configuring the following parameters:

-	drx-onDurationTimer: the duration at the beginning of a DRX Cycle;

-	drx-SlotOffset: the delay before starting the drx-onDurationTimer;

-	drx-InactivityTimer: the duration after the PDCCH occasion in which a PDCCH indicates a new UL or DL transmission for the MAC entity;

-	drx-RetransmissionTimerDL (per DL HARQ process except for the broadcast process): the maximum duration until a DL retransmission is received;

-	drx-RetransmissionTimerUL (per UL HARQ process): the maximum duration until a grant for UL retransmission is received;

-	drx-LongCycleStartOffset: the Long DRX cycle and drx-StartOffset which defines the subframe where the Long and Short DRX Cycle starts;

-	drx-ShortCycle (optional): the Short DRX cycle;

-	drx-ShortCycleTimer (optional): the duration the UE shall follow the Short DRX cycle;

-	drx-HARQ-RTT-TimerDL (per DL HARQ process except for the broadcast process): the minimum duration before a DL assignment for HARQ retransmission is expected by the MAC entity;

-	drx-HARQ-RTT-TimerUL (per UL HARQ process): the minimum duration before a UL HARQ retransmission grant is expected by the MAC entity.

When DRX is configured, the Active Time includes the time while:

-	drx-onDurationTimer or drx-InactivityTimer or drx-RetransmissionTimerDL or drx-RetransmissionTimerUL or ra-ContentionResolutionTimer (as described in clause 5.1.5) is running; or

-	a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4); or

-	a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clause 5.1.4).

When DRX is configured, the MAC entity shall:

...

1>	if drx-InactivityTimer expires or a DRX Command MAC CE is received:

2>	if the Short DRX cycle is configured:

3>	start or restart drx-ShortCycleTimer in the first symbol after the expiry of drx-InactivityTimer or in the first symbol after the end of DRX Command MAC CE reception;

3>	use the Short DRX Cycle.

2>	else:

3>	use the Long DRX cycle.

1>	if drx-ShortCycleTimer expires:

2>	use the Long DRX cycle.

1>	if a Long DRX Command MAC CE is received:

2>	stop drx-ShortCycleTimer;

2>	use the Long DRX cycle.

1>	if the Short DRX Cycle is used, and [(SFN × 10) + subframe number] modulo (drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle); or

1>	if the Long DRX Cycle is used, and [(SFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset:

2>	start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe.

7.1.1.5.3.3	Test description

7.1.1.5.3.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that set to return no data in uplink.

7.1.1.5.3.3.2	Test procedure sequence

For FDD, NormalSLT (current SFN, current sub-frame, current slot, y) = y; For TDD, NormalSLT (current SFN, current slot, y) counts the minimum number of normal slots needed to cover y number of PDCCH-occasions(slots) until next PDCCH-occasion(slot) available, starting from current slot on current Subframe.

Table 7.1.1.5.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | SS transmits NR RRCReconfiguration message to configure specific DRX parameters for SpCell (Note1). | <-- | - | - | - |
| 2 | The UE transmitNR RRCReconfigurationComplete messages. (Note 2) | --> | - | - | - |
| 3 | In the first PDCCH occasion, after the drx-SlotOffset when the drx-onDurationTimer is running, the SS indicates the transmission of a DL MAC PDU on the PDCCH. (Note 3)(Note 4)(Note 5) | <-- | MAC PDU | - | - |
| 4 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 3? | --> | HARQ ACK | - | - |
| 5 | At least drx-InactivityTimer after the transmission of the MAC PDU in Step 3 has been indicated (This means the next DRX cycle or later after Step 1) in the last PDCCH occasion while the drx-onDurationTimer is still running according to [(SFN * 10) + subframe number] modulo drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle)), the SS indicates the transmission a DL MAC PDU on the PDDCH. | <-- | MAC PDU | - | - |
| 6 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 5? | --> | HARQ ACK | 1 | P |
| 7 | SS waits for drx-ShortCycleTimer to expire. | - | - | - | - |
| 8 | In the first PDCCH occasion after the drx-SlotOffset when the drx-onDurationTimer of drx-LongCycle is running, the SS indicates the transmission of a DL MAC PDU on the PDCCH. | <-- | MAC PDU | - | - |
| 9 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 8? | --> | HARQ ACK | 2 | P |
| Note 1: For EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration 36.508 [7], Table 4.6.1-8 using condition EN-DC_EmbedNR_RRCRecon. Note 2: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. Note 3: The drx-InactivityTimer is started in the first symbol after the end of the PDCCH reception where DL new transmission is indicated. Note 4: When the drx-InactivityTimer expires, UE starts drx-ShortCycleTimer in the first symbol after the expiry of drx-InactivityTimer. Note 5: The SS assumes that the UE starts in long DRX after configuration. |  |  |  |  |  |

7.1.1.5.3.3.3	Specific message contents

Table 7.1.1.5.3.3.3-1: RRCReconfiguration (step 1, Table 7.1.1.5.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig |  | NR |
| secondaryCellGroup | CellGroupConfig |  | EN-DC |
| nonCriticalExtension SEQUENCE { |  |  | NR |
| masterCellGroup | CellGroupConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.5.3.3.3-2: CellGroupConfig (Table 7.1.1.5.3.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| mac-CellGroupConfig SEQUENCE { |  |  |  |
| drx-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| drx-onDurationTimer | ms20 |  |  |
| drx-InactivityTimer | ms10 |  |  |
| drx-HARQ-RTT-TimerDL | 56 |  |  |
| drx-HARQ-RTT-TimerUL | 56 |  |  |
| drx-RetransmissionTimerDL | sl80 |  |  |
| drx-RetransmissionTimerUL | sl80 |  |  |
| drx-LongCycleStartOffset CHOICE { |  |  |  |
| ms640 | 7 |  |  |
| } |  |  |  |
| shortDRX SEQUENCE { |  |  |  |
| drx-ShortCycle | ms80 |  |  |
| drx-ShortCycleTimer | 7 |  |  |
| } |  |  |  |
| drx-SlotOffset | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 7.1.1.5.4	DRX operation / Short cycle configured / DRX command MAC control element reception

7.1.1.5.4.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { Short DRX cycle is configured and a DRX Command MAC control element is received }

then { UE successfully decodes the MAC control PDU }

}

(2)

with { UE in RRC_CONNECTED state }

ensure that {

when { Short DRX cycle is configured and the HARQ RTT Timer is running and a DRX Command MAC control element is received }

then { UE continues running the HARQ RTT timer }

}

(3)

with { UE in RRC_CONNECTED state }

ensure that {

when { Short DRX cycle is configured and the drx-RetransmissionTimer-DL is running and a DRX Command MAC control element is received }

then { UE continues running the drx-RetransmissionTimer-DL and monitors the PDCCH }

}

7.1.1.5.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.321, clause 5.7. Unless otherwise stated these are Rel-15 requirements.

[TS 38.321, clause 5.7]

The MAC entity may be configured by RRC with a DRX functionality that controls the UE's PDCCH monitoring activity for the MAC entity's C-RNTI, CS-RNTI, INT-RNTI, SFI-RNTI, SP-CSI-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, and TPC-SRS-RNTI. When using DRX operation, the MAC entity shall also monitor PDCCH according to requirements found in other subclauses of this specification. When in RRC_CONNECTED, if DRX is configured, for all the activated Serving Cells, the MAC entity may monitor the PDCCH discontinuously using the DRX operation specified in this subclause; otherwise the MAC entity shall monitor the PDCCH continuously.

RRC controls DRX operation by configuring the following parameters:

-	drx-onDurationTimer: the duration at the beginning of a DRX Cycle;

-	drx-SlotOffset: the delay before starting the drx-onDurationTimer;

-	drx-InactivityTimer: the duration after the PDCCH occasion in which a PDCCH indicates a new UL or DL transmission for the MAC entity;

-	drx-RetransmissionTimerDL (per DL HARQ process except for the broadcast process): the maximum duration until a DL retransmission is received;

-	drx-RetransmissionTimerUL (per UL HARQ process): the maximum duration until a grant for UL retransmission is received;

-	drx-LongCycleStartOffset: the Long DRX cycle and drx-StartOffset which defines the subframe where the Long and Short DRX Cycle starts;

-	drx-ShortCycle (optional): the Short DRX cycle;

-	drx-ShortCycleTimer (optional): the duration the UE shall follow the Short DRX cycle;

-	drx-HARQ-RTT-TimerDL (per DL HARQ process except for the broadcast process): the minimum duration before a DL assignment for HARQ retransmission is expected by the MAC entity;

-	drx-HARQ-RTT-TimerUL (per UL HARQ process): the minimum duration before a UL HARQ retransmission grant is expected by the MAC entity.

When a DRX cycle is configured, the Active Time includes the time while:

-	drx-onDurationTimer or drx-InactivityTimer or drx-RetransmissionTimerDL or drx-RetransmissionTimerUL or ra-ContentionResolutionTimer (as described in subclause 5.1.5) is running; or

-	a Scheduling Request is sent on PUCCH and is pending (as described in subclause 5.4.4); or

-	a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in subclause 5.1.4).

When DRX is configured, the MAC entity shall:

1>	if a MAC PDU is received in a configured downlink assignment:

2>	start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback;

2>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process.

1>	if a MAC PDU is transmitted in a configured uplink grant:

2>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first repetition of the corresponding PUSCH transmission;

2>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process.

1>	if a drx-HARQ-RTT-TimerDL expires:

2>	if the data of the corresponding HARQ process was not successfully decoded:

3>	start the drx-RetransmissionTimerDL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerDL.

1>	if a drx-HARQ-RTT-TimerUL expires:

2>	start the drx-RetransmissionTimerUL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerUL.

1>	if a DRX Command MAC CE or a Long DRX Command MAC CE is received:

2>	stop drx-onDurationTimer;

2>	stop drx-InactivityTimer.

1>	if drx-InactivityTimer expires or a DRX Command MAC CE is received:

2>	if the Short DRX cycle is configured:

3>	start or restart drx-ShortCycleTimer in the first symbol after the expiry of drx-InactivityTimer or in the first symbol after the end of DRX Command MAC CE reception;

3>	use the Short DRX Cycle.

2>	else:

3>	use the Long DRX cycle.

1>	if drx-ShortCycleTimer expires:

2>	use the Long DRX cycle.

1>	if a Long DRX Command MAC CE is received:

2>	stop drx-ShortCycleTimer;

2>	use the Long DRX cycle.

1>	if the Short DRX Cycle is used, and [(SFN × 10) + subframe number] modulo (drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle); or

1>	if the Long DRX Cycle is used, and [(SFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset:

2>	start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe.

1>	if the MAC entity is in Active Time:

2>	monitor the PDCCH;

2>	if the PDCCH indicates a DL transmission:

3>	start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback;

3>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process.

2>	if the PDCCH indicates a UL transmission:

3>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first repetition of the corresponding PUSCH transmission;

3>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process.

2>	if the PDCCH indicates a new transmission (DL or UL):

3>	start or restart drx-InactivityTimer in the first symbol after the end of the PDCCH reception.

1>	in current symbol n, if the MAC entity would not be in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this subclause:

2>	not transmit periodic SRS and semi-persistent SRS defined in TS 38.214 [7].

1>	if CSI masking (csi-Mask) is setup by upper layers:

2>	in current symbol n, if onDurationTimer would not be running considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this subclause:

3>	not report CSI on PUCCH.

1>	else:

2>	in current symbol n, if the MAC entity would not be in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this subclause:

3>	not report CSI on PUCCH and semi-persistent CSI on PUSCH.

Regardless of whether the MAC entity is monitoring PDCCH or not, the MAC entity transmits HARQ feedback, aperiodic CSI on PUSCH, and aperiodic SRS defined in TS 38.214 [7] when such is expected.

The MAC entity needs not to monitor the PDCCH if it is not a complete PDCCH occasion (e.g. the Active Time starts or ends in the middle of a PDCCH occasion).

7.1.1.5.4.3	Test description

7.1.1.5.4.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that set to return no data in uplink.

7.1.1.5.4.3.2	Test procedure sequence

For FDD, NormalSLT(current SFN, current subframe, current slot, y)=y; For TDD, NormalSLT(current SFN, current subframe, current slot, y) counts the minimum number of normal slots needed to cover y number of PDCCH-occasions (slots) until next PDCCH-occasion (slot) available, starting from current slot on current SFN.

Table 7.1.1.5.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits NR RRCReconfigurationmessage to configure specific DRX parameters for NR Cell. (Note 1) | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits NR RRCReconfigurationComplete message. (Note 2) | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | In a PDCCH occasion which is X subframes before the PDCCH occasion in which the drx-onDurationTimer expires, with X < drx-onDurationTimer-1, the SS indicates the transmission of a DL MAC PDU on the PDCCH. (Note 7) | <-- | MAC PDU | - | - |
| 4 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 3? | --> | HARQ ACK | 1 | P |
| 5 | In a PDCCH occasion before the drx-onDurationTimer expires, the SS indicates the transmission of a DL MAC PDU on the PDCCH. The SS transmits a DL MAC PDU with DRX MAC Control element. UE successfully decodes the MAC PDU. | <-- | MAC PDU (DRX MAC Control element) | - | - |
| 6 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 5? | --> | HARQ ACK | 1 | P |
| 6A | In a PDCCH occasion before the short DRX cycle ends, the SS indicates the transmission of a DL MAC PDU on the PDCCH. The SS transmits a DL MAC PDU | <-- | MAC PDU |  |  |
| 6B | Check: Does the UE transmit a HARQ ACK in step 6B? | --> | HARQ ACK | 1 | F |
| 7 | On the next or later DRX cycle than the one used for Step 3 and on a PDCCH occasion which is X PDCCH sub frames before the PDCCH occasion in which the onDurationTimer expires, with X < drx-onDurationTimer, the SS indicates the transmission of a DL MAC PDU. The SS transmits an invalid MAC PDU. (Note 3) | <-- | MAC PDU | - | - |
| 8 | Check: Does the UE transmit a HARQ NACK for the DL MAC PDU in Step 7? | --> | HARQ NACK | 2,3 | P |
| 8A | In a PDCCH occasion before the Drx-HARQ-RTT-TimerDL for the MAC PDU indicated in Step 7 expires, the SS indicates the transmission of a DL MAC PDU on the PDCCH. The SS transmits a DL MAC PDU with DRX MAC Control element. | <-- | MAC PDU(DRX MAC Control element) |  |  |
| 8B | Check: Does the UE transmit a HARQ ACK? | --> | HARQ ACK | 2,3 | P |
| 9 | In a PDCCH occasion which is Z slots before the slot in which the drx-RetransmissionTimerDL for the DL MAC PDU in Step 7 expires, with 1 <Z< drx-RetransmissionTimerDL, the SS indicates the transmission of a DL MAC PDU. The SS transmits a DL MAC PDU with DRX MAC Control element. | <-- | MAC PDU(DRX MAC Control element) | - | - |
| 10 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 9? | --> | HARQ ACK | 2,3,1 | P |
| 11 | In the last PDCCH occasion when the drx-RetransmissionTimerDL for the DL MAC PDU indicated on the PDCCH in Step 7 is still running, the SS indicates the transmission of a DL MAC PDU. | <-- | MAC PDU | - | - |
| 12 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 11? | --> | HARQ ACK | 2,3 | P |
| Note 1: For EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration. Note 2: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. Note 3: Invalid MAC PDU is a MAC PDU that fails the CRC check. Note 4: All DL MAC PDUs are transmitted with the NDI set on the PDCCH. Note 5: Timer tolerances for the MAC DRX related timers measured in PDCCH occasions (slots). These timers are: drx-InactivityTimer, drx-RetransmissionTimer, Drx-HARQ-RTT-TimerDL. Note 6: K is the time for given PDSCH to HARQ feedback of PUCCH and shall be shorter than drx-InactivityTimer. In this TC, the DCI format should be configured to not include the PDSCH-to-HARQ-timing-indicator field. When the UE schedules a PDSCH reception over a number of symbols where the last symbol is within slot n-k, the UE shall provide corresponding HARQ-ACK information in a PUCCH transmission within slot n-k+4 according to TS 38.321 clause 9.2.3. Thus, the maximum value of K is 4 slots in this test case. Note 7: The SS assumes that the UE starts in long DRX after configuration. |  |  |  |  |  |

7.1.1.5.4.3.3	Specific message contents

Table 7.1.1.5.4.3.3-1: RRCReconfiguration (Step 1, Table 7.1.1.5.4.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |  |  |  |
| radioBearerConfig |  | Not present |  |  |  |  |  |
| secondaryCellGroup |  | CellGroupConfig |  | OCTET STRING (CONTAINING CellGroupConfig) |  | EN-DC |  |
| nonCriticalExtension |  | Not present |  |  |  | EN-DC |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |  |  | NR |  |
| masterCellGroup |  | CellGroupConfig |  | OCTET STRING (CONTAINING CellGroupConfig) |  |  |  |
| dedicatedNAS-MessageList |  | Not present |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 7.1.1.5.4.3.3-2: CellGroupConfig (Table 7.1.1.5.4.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| mac-CellGroupConfig SEQUENCE { |  |  |  |
| drx-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| drx-onDurationTimer | ms40 |  |  |
| drx-InactivityTimer | Ms10 |  |  |
| drx-HARQ-RTT-TimerDL | 56 |  |  |
| drx-HARQ-RTT-TimerUL | 56 |  |  |
| drx-RetransmissionTimerDL | Sl80 |  |  |
| drx-RetransmissionTimerUL | Sl80 |  |  |
| drx-LongCycleStartOffset CHOICE { |  |  |  |
| ms640 | 7 |  |  |
| } |  |  |  |
| shortDRX SEQUENCE { |  |  |  |
| drx-ShortCycle | ms80 |  |  |
| drx-ShortCycleTimer | 7 |  |  |
| } |  |  |  |
| drx-SlotOffset | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 7.1.1.5.5	DRX operation / Short cycle configured / Long DRX command MAC control element reception

7.1.1.5.5.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { short DRX cycle is configured and a long DRX Command MAC control element is received }

then { UE successfully decodes the MAC control PDU }

}

(2)

with { UE in RRC_CONNECTED state }

ensure that {

when { Long DRX cycle and drx-SlotOffset is configured and [(SFN * 10) + subframe number] modulo drx-LongCycle} = drxStartOffset }

then { UE starts the OnDurationTimer after drx-SlotOffset and monitors PDCCH for OnDurationTimer PDCCH-subframes }

}

7.1.1.5.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.321, clause 5.7. Unless otherwise stated these are Rel-15 requirements.

[TS 38.321, clause 5.7]

The MAC entity may be configured by RRC with a DRX functionality that controls the UE's PDCCH monitoring activity for the MAC entity's C-RNTI, CS-RNTI, INT-RNTI, SFI-RNTI, SP-CSI-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, and TPC-SRS-RNTI. When using DRX operation, the MAC entity shall also monitor PDCCH according to requirements found in other clauses of this specification. When in RRC_CONNECTED, if DRX is configured, for all the activated Serving Cells, the MAC entity may monitor the PDCCH discontinuously using the DRX operation specified in this clause; otherwise the MAC entity shall monitor the PDCCH as specified in TS 38.213 [6].

RRC controls DRX operation by configuring the following parameters:

-	drx-onDurationTimer: the duration at the beginning of a DRX Cycle;

-	drx-SlotOffset: the delay before starting the drx-onDurationTimer;

-	drx-InactivityTimer: the duration after the PDCCH occasion in which a PDCCH indicates a new UL or DL transmission for the MAC entity;

-	drx-RetransmissionTimerDL (per DL HARQ process except for the broadcast process): the maximum duration until a DL retransmission is received;

-	drx-RetransmissionTimerUL (per UL HARQ process): the maximum duration until a grant for UL retransmission is received;

-	drx-LongCycleStartOffset: the Long DRX cycle and drx-StartOffset which defines the subframe where the Long and Short DRX Cycle starts;

-	drx-ShortCycle (optional): the Short DRX cycle;

-	drx-ShortCycleTimer (optional): the duration the UE shall follow the Short DRX cycle;

-	drx-HARQ-RTT-TimerDL (per DL HARQ process except for the broadcast process): the minimum duration before a DL assignment for HARQ retransmission is expected by the MAC entity;

-	drx-HARQ-RTT-TimerUL (per UL HARQ process): the minimum duration before a UL HARQ retransmission grant is expected by the MAC entity.

When a DRX cycle is configured, the Active Time includes the time while:

-	drx-onDurationTimer or drx-InactivityTimer or drx-RetransmissionTimerDL or drx-RetransmissionTimerUL or ra-ContentionResolutionTimer (as described in clause 5.1.5) is running; or

-	a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4); or

-	a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clause 5.1.4).

When DRX is configured, the MAC entity shall:

1>	if a MAC PDU is received in a configured downlink assignment:

2>	start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback;

2>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process.

1>	if a MAC PDU is transmitted in a configured uplink grant:

2>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first repetition of the corresponding PUSCH transmission;

2>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process.

1>	if a drx-HARQ-RTT-TimerDL expires:

2>	if the data of the corresponding HARQ process was not successfully decoded:

3>	start the drx-RetransmissionTimerDL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerDL.

1>	if a drx-HARQ-RTT-TimerUL expires:

2>	start the drx-RetransmissionTimerUL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerUL.

1>	if a DRX Command MAC CE or a Long DRX Command MAC CE is received:

2>	stop drx-onDurationTimer;

2>	stop drx-InactivityTimer.

1>	if drx-InactivityTimer expires or a DRX Command MAC CE is received:

2>	if the Short DRX cycle is configured:

3>	start or restart drx-ShortCycleTimer in the first symbol after the expiry of drx-InactivityTimer or in the first symbol after the end of DRX Command MAC CE reception;

3>	use the Short DRX Cycle.

2>	else:

3>	use the Long DRX cycle.

1>	if drx-ShortCycleTimer expires:

2>	use the Long DRX cycle.

1>	if a Long DRX Command MAC CE is received:

2>	stop drx-ShortCycleTimer;

2>	use the Long DRX cycle.

1>	if the Short DRX Cycle is used, and [(SFN × 10) + subframe number] modulo (drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle); or

1>	if the Long DRX Cycle is used, and [(SFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset:

2>	start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe.

1>	if the MAC entity is in Active Time:

2>	monitor the PDCCH as specified in TS 38.213 [6];

2>	if the PDCCH indicates a DL transmission:

3>	start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback;

3>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process.

2>	if the PDCCH indicates a UL transmission:

3>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first repetition of the corresponding PUSCH transmission;

3>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process.

2>	if the PDCCH indicates a new transmission (DL or UL):

3>	start or restart drx-InactivityTimer in the first symbol after the end of the PDCCH reception.

1>	in current symbol n, if the MAC entity would not be in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause:

2>	not transmit periodic SRS and semi-persistent SRS defined in TS 38.214 [7];

2>	not report CSI on PUCCH and semi-persistent CSI configured on PUSCH.

1>	if CSI masking (csi-Mask) is setup by upper layers:

2>	in current symbol n, if drx-onDurationTimer would not be running considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause:

3>	not report CSI on PUCCH.

NOTE:	If a UE multiplexes a CSI configured on PUCCH with other overlapping UCI(s) according to the procedure specified in TS 38.213 [6] subclause 9.2.5 and this CSI multiplexed with other UCI(s) would be reported on a PUCCH resource outside DRX Active Time, it is up to UE implementation whether to report this CSI multiplexed with other UCI(s).

Regardless of whether the MAC entity is monitoring PDCCH or not, the MAC entity transmits HARQ feedback, aperiodic CSI on PUSCH, and aperiodic SRS defined in TS 38.214 [7] when such is expected.

The MAC entity needs not to monitor the PDCCH if it is not a complete PDCCH occasion (e.g. the Active Time starts or ends in the middle of a PDCCH occasion).

7.1.1.5.5.3	Test Description

7.1.1.5.5.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that set to return no data in uplink.

7.1.1.5.5.3.2	Test procedure sequence

For FDD, NormalSLT=(current SFN, current sub-frame, current slot, y)=y; For TDD, NormalSLT(current SFN, current slot, y) counts the minimum number of normal slots needed to cover y number of PDCCH-occasions (slots) until next PDCCH-occasion(slot) available, starting from current slot on current SFN.

Table 7.1.1.5.5.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits NR RRCReconfiguration message to configure specific DRX parameters for NR Cell. (Note 1) | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits NR RRCReconfigurationComplete message. (Note 2) | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | In a PDCCH occasion when the drx-onDurationTimer is running, the SS indicates the transmission of a DL MAC PDU on the PDCCH. The SS transmits an valid MAC PDU. (Note 3) | <-- | MAC PDU | - | - |
| 4 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 3? | --> | HARQ ACK | 1 | P |
| 4A | Wait for the expiry of drx-InactivityTimer to ensure that UE starts the Short DRX Cycle. | - | - | - | - |
| 5 | In a PDCCH occasion before the drx-onDurationTimer expires, the SS indicates the transmission of a DL MAC PDU on the PDCCH. The SS transmits a DL MAC PDU with Long DRX MAC Control element. UE successfully decodes the MAC PDU. | <-- | MAC PDU (Long DRX MAC Control element) | - | - |
| 6 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 5? | --> | HARQ ACK | 1 | P |
| 7 | In the first PDCCH occasion, after the drx-SlotOffset when the drx-onDurationTimer is running according to [(SFN * 10) + subframe number] modulo drx-LongCycle) = (drx-StartOffset) modulo (drx-LongCycle)), the SS indicates the transmission of a DL MAC PDU on the PDCCH. | <-- | MAC PDU | - | - |
| 8 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 7? | --> | HARQ ACK | 2 | P |
| 9 | At least drx-InactivityTimer PDCCH occasions after the transmission of the MAC PDU in Step 7 has been indicated (This means the next DRX cycle or later after Step 7) in the last PDCCH occasion while the drx-onDurationTimer is still running, the SS indicates the transmission a DL MAC PDU on the PDDCH. (Note 7) | <-- | MAC PDU | - | - |
| 10 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 9? | --> | HARQ ACK | 2 | P |
| Note 1: For EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration 36.508 [7], Table 4.6.1-8 using condition EN-DC_EmbedNR_RRCRecon. Note 2: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. Note 3: The SS assumes that the UE starts in long DRX after configuration. Note 4: All DL MAC PDUs are transmitted with the NDI set on the PDCCH. Note 5: Timer tolerances for the MAC DRX related timers measured in PDCCH occasions (slots). These timers are: drx-InactivityTimer, drx-RetransmissionTimer, Drx-HARQ-RTT-TimerDL. Note 6: Void Note 7: The drx-InactivityTimer is started in the next PDCCH occasion of the PDCCH occasion where DL new transmission is indicated. |  |  |  |  |  |

7.1.1.5.5.3.3	Specific message contents

Table 7.1.1.5.5.3.3-1: RRCReconfiguration (Step 1, Table 7.1.1.5.5.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig with conditions SRB2 and DRB1 |  | NR |
| secondaryCellGroup | CellGroupConfig |  | EN-DC |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig with condition SRB2_DRB1 |  | NR |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.5.5.3.3-2: CellGroupConfig (Table 7.1.1.5.5.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| mac-CellGroupConfig SEQUENCE { |  |  |  |
| drx-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| drx-onDurationTimer | ms40 |  |  |
| drx-InactivityTimer | ms10 |  |  |
| drx-HARQ-RTT-TimerDL | 56 |  |  |
| drx-HARQ-RTT-TimerUL | 56 |  |  |
| drx-RetransmissionTimerDL | sl80 |  |  |
| drx-RetransmissionTimerUL | sl80 |  |  |
| drx-LongCycleStartOffset CHOICE { |  |  |  |
| ms640 | 7 |  |  |
| } |  |  |  |
| shortDRX SEQUENCE { |  |  |  |
| drx-ShortCycle | ms80 |  |  |
| drx-ShortCycleTimer | 7 |  |  |
| } |  |  |  |
| drx-SlotOffset | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 7.1.1.5.6	NR NTN / DRX operation / Short cycle not configured / HARQ RTT

7.1.1.5.6.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with long DRX cycle configured and the corresponding HARQ process is configured with HARQ feedback enabled }

ensure that {

when { HARQ-RTT-TimerDL-NTN expires in the PDCCH Occasion and the data in the soft buffer of the corresponding HARQ process was not successfully decoded }

then { UE starts the drx-RetransmissionTimerDL for the corresponding HARQ process in the first symbol after the expiry of HARQ-RTT-TimerDL-NTN }

}

(2)

with { UE in RRC_CONNECTED state with long DRX cycle configured and the corresponding HARQ process is configured as HARQModeA }

ensure that {

when { HARQ-RTT-TimerUL-NTN expires in the PDCCH Occasion }

then { UE starts the drx-RetransmissionTimerUL for the corresponding HARQ process in the first symbol after the expiry of HARQ-RTT-TimerUL-NTN }

}

7.1.1.5.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.321, clause 5.7. Unless otherwise stated these are Rel-17 requirements.

[TS 38.321, clause 5.7]

The MAC entity may be configured by RRC with a DRX functionality that controls the UE's PDCCH monitoring activity for the MAC entity's C-RNTI, CI-RNTI, CS-RNTI, INT-RNTI, SFI-RNTI, SP-CSI-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, TPC-SRS-RNTI, AI-RNTI, SL-RNTI, SL-CS-RNTI and SL Semi-Persistent Scheduling V-RNTI. When using DRX operation, the MAC entity shall also monitor PDCCH according to requirements found in other clauses of this specification. When in RRC_CONNECTED, if DRX is configured, for all the activated Serving Cells, the MAC entity may monitor the PDCCH discontinuously using the DRX operation specified in this clause; otherwise the MAC entity shall monitor the PDCCH as specified in TS 38.213 [6].

NOTE 1:	Void

RRC controls DRX operation by configuring the following parameters:

-	drx-onDurationTimer: the duration at the beginning of a DRX cycle;

-	drx-SlotOffset: the delay before starting the drx-onDurationTimer;

-	drx-InactivityTimer: the duration after the PDCCH occasion in which a PDCCH indicates a new UL, DL or SL transmission for the MAC entity;

-	drx-RetransmissionTimerDL (per DL HARQ process except for the broadcast process): the maximum duration until a DL retransmission is received;

-	drx-RetransmissionTimerUL (per UL HARQ process): the maximum duration until a grant for UL retransmission is received;

-	drx-LongCycleStartOffset: the Long DRX cycle and drx-StartOffset which defines the subframe where the Long and Short DRX cycle starts;

-	drx-ShortCycle (optional): the Short DRX cycle;

-	drx-ShortCycleTimer (optional): the duration the UE shall follow the Short DRX cycle;

-	drx-HARQ-RTT-TimerDL (per DL HARQ process except for the broadcast process): the minimum duration before a DL assignment for HARQ retransmission is expected by the MAC entity;

-	drx-HARQ-RTT-TimerUL (per UL HARQ process): the minimum duration before a UL HARQ retransmission grant is expected by the MAC entity;

-	drx-RetransmissionTimerSL (per sidelink process): the maximum duration until a grant for SL retransmission is received;

-	drx-HARQ-RTT-TimerSL (per sidelink process): the minimum duration before an SL retransmission grant is expected by the MAC entity;

-	drx-LastTransmissionUL (optional): the configuration to start drx-HARQ-RTT-TimerUL after the last transmission within a bundle;

-	ps-Wakeup (optional): the configuration to start associated drx-onDurationTimer in case DCP is monitored but not detected;

-	ps-TransmitOtherPeriodicCSI (optional): the configuration to report periodic CSI that is not L1-RSRP on PUCCH during the time duration indicated by drx-onDurationTimer in case DCP is configured but associated drx-onDurationTimer is not started;

-	ps-TransmitPeriodicL1-RSRP (optional): the configuration to transmit periodic CSI that is L1-RSRP on PUCCH during the time duration indicated by drx-onDurationTimer in case DCP is configured but associated drx-onDurationTimer is not started;

-	downlinkHARQ-FeedbackDisabled (optional): the configuration to disable HARQ feedback per DL HARQ process;

-	uplinkHARQ-Mode (optional): the configuration to set HARQmodeA or HARQmodeB per UL HARQ process.

Serving Cells of a MAC entity may be configured by RRC in two DRX groups with separate DRX parameters. When RRC does not configure a secondary DRX group, there is only one DRX group and all Serving Cells belong to that one DRX group. When two DRX groups are configured, each Serving Cell is uniquely assigned to either of the two groups. The DRX parameters that are separately configured for each DRX group are: drx-onDurationTimer, drx-InactivityTimer. The DRX parameters that are common to the DRX groups are: drx-SlotOffset, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-LongCycleStartOffset, drx-ShortCycle (optional), drx-ShortCycleTimer (optional), drx-HARQ-RTT-TimerDL, and drx-HARQ-RTT-TimerUL.

When DRX is configured, the Active Time for Serving Cells in a DRX group includes the time while:

-	drx-onDurationTimer or drx-InactivityTimer configured for the DRX group is running; or

-	drx-RetransmissionTimerDL, drx-RetransmissionTimerUL or drx-RetransmissionTimerSL is running on any Serving Cell in the DRX group; or

-	ra-ContentionResolutionTimer (as described in clause 5.1.5) or msgB-ResponseWindow (as described in clause 5.1.4a) is running; or

-	a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4 or 5.22.1.5). If this Serving Cell is part of a non-terrestrial network, the Active Time is started after the Scheduling Request transmission that is performed when the SR_COUNTER is 0 for all the SR configurations with pending SR(s) plus the UE-gNB RTT; or

-	a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clauses 5.1.4 and 5.1.4a).

The following MAC timers are used for DRX operation in a non-terrestrial network:

-	HARQ-RTT-TimerDL-NTN (per DL HARQ process configured with HARQ feedback enabled): the minimum duration before a DL assignment for HARQ retransmission is expected by the MAC entity;

-	HARQ-RTT-TimerUL-NTN (per UL HARQ process configured with HARQModeA): the minimum duration before a UL HARQ retransmission grant is expected by the MAC entity.

When DRX is configured, the MAC entity shall:

1>	if a MAC PDU is received in a configured downlink assignment for unicast:

2>	if this Serving Cell is configured with downlinkHARQ-FeedbackDisabled:

3>	if the corresponding HARQ process is configured with HARQ feedback enabled:

4>	set HARQ-RTT-TimerDL-NTN for the corresponding HARQ process equal to drx-HARQ-RTT-TimerDL plus the latest available UE-gNB RTT value;

4>	start the HARQ-RTT-TimerDL-NTN for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

2>	else:

3>	start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

NOTE 1a:	Void.

NOTE 1b:	Void.

2>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process;

2>	stop the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process.

1>	if a MAC PDU is transmitted in a configured uplink grant and LBT failure indication is not received from lower layers:

2>	if this Serving Cell is configured with uplinkHARQ-Mode:

3>	if the corresponding HARQ process is configured as HARQModeA:

4>	set HARQ-RTT-TimerUL-NTN for the corresponding HARQ process equal to drx-HARQ-RTT-TimerUL plus the latest available UE-gNB RTT value;

4>	if drx-LastTransmissionUL is configured:

5>	start the HARQ-RTT-TimerUL-NTN for the corresponding HARQ process in the first symbol after the end of the last transmission (within a bundle) of the corresponding PUSCH transmission.

4>	else:

5>	start the HARQ-RTT-TimerUL-NTN for the corresponding HARQ process in the first symbol after the end of the first transmission (within a bundle) of the corresponding PUSCH transmission.

2>	else:

3>	if drx-LastTransmissionUL is configured:

4>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the last transmission (within a bundle) of the corresponding PUSCH transmission.

3>	else:

4>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first transmission (within a bundle) of the corresponding PUSCH transmission.

2>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process at the first transmission (within a bundle) of the corresponding PUSCH transmission.

1>	if a drx-HARQ-RTT-TimerDL expires:

2>	if the data of the corresponding HARQ process was not successfully decoded:

3>	start the drx-RetransmissionTimerDL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerDL.

1>	if a HARQ-RTT-TimerDL-NTN expires:

2>	if the data of the corresponding HARQ process was not successfully decoded:

3>	start the drx-RetransmissionTimerDL for the corresponding HARQ process in the first symbol after the expiry of HARQ-RTT-TimerDL-NTN.

1>	if a drx-HARQ-RTT-TimerUL expires:

2>	start the drx-RetransmissionTimerUL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerUL.

1>	if a HARQ-RTT-TimerUL-NTN expires:

2>	start the drx-RetransmissionTimerUL for the corresponding HARQ process in the first symbol after the expiry of HARQ-RTT-TimerUL-NTN.

1>	if a DRX Command MAC CE indicated by PDCCH addressed to C-RNTI or CS-RNTI, or by a configured downlink assignment for unicast transmission or a Long DRX Command MAC CE is received:

2>	stop drx-onDurationTimer for each DRX group;

2>	stop drx-InactivityTimer for each DRX group.

1>	if drx-InactivityTimer for a DRX group expires:

2>	if the Short DRX cycle is configured:

3>	start or restart drx-ShortCycleTimer for this DRX group in the first symbol after the expiry of drx-InactivityTimer;

3>	use the Short DRX cycle for this DRX group.

2>	else:

3>	use the Long DRX cycle for this DRX group.

1>	if the Long DRX cycle is used for a DRX group, and [(SFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset:

2>	if DCP monitoring is configured for the active DL BWP as specified in TS 38.213 [6], clause 10.3:

3>	if DCP indication associated with the current DRX cycle received from lower layer indicated to start drx-onDurationTimer, as specified in TS 38.213 [6]; or

3>	if all DCP occasion(s) in time domain, as specified in TS 38.213 [6], associated with the current DRX cycle occurred in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to start of the last DCP occasion, or during a measurement gap, or when the MAC entity monitors for a PDCCH transmission on the search space indicated by recoverySearchSpaceId of the SpCell identified by the C-RNTI while the ra-ResponseWindow is running (as specified in clause 5.1.4); or

3>	if ps-Wakeup is configured with value true and DCP indication associated with the current DRX cycle has not been received from lower layers:

4>	start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe.

2>	else:

3>	start drx-onDurationTimer for this DRX group after drx-SlotOffset from the beginning of the subframe.

NOTE 2:	In case of unaligned SFN across carriers in a cell group, the SFN of the SpCell is used to calculate the DRX duration.

1>	if a DRX group is in Active Time:

2>	monitor the PDCCH on the Serving Cells in this DRX group as specified in TS 38.213 [6];

2>	if the PDCCH indicates a DL transmission; or

2>	if the PDCCH indicates a one-shot HARQ feedback as specified in clause 9.1.4 of TS 38.213 [6]; or

2>	if the PDCCH indicates a retransmission of HARQ feedback as specified in clause 9.1.5 of TS 38.213 [6]:

3>	if this Serving Cell is configured with downlinkHARQ-FeedbackDisabled:

4>	if the corresponding HARQ process is configured with HARQ feedback enabled:

5>	set HARQ-RTT-TimerDL-NTN for the corresponding HARQ process equal to drx-HARQ-RTT-TimerDL plus the latest available UE-gNB RTT value;

5>	start the HARQ-RTT-TimerDL-NTN for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

3>	else:

4>	start or restart the drx-HARQ-RTT-TimerDL for the corresponding HARQ process(es) whose HARQ feedback is reported in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

NOTE 3:	When HARQ feedback is postponed by PDSCH-to-HARQ_feedback timing indicating an inapplicable k1 value, as specified in TS 38.213 [6], the corresponding transmission opportunity to send the DL HARQ feedback is indicated in a later PDCCH requesting the HARQ-ACK feedback.

3>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process(es) whose HARQ feedback is reported;

3>	stop the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process;

3>	if the PDSCH-to-HARQ_feedback timing indicate an inapplicable k1 value as specified in TS 38.213 [6]:

4>	start the drx-RetransmissionTimerDL in the first symbol after the (end of the last) PDSCH transmission (within a bundle) for the corresponding HARQ process.

2>	if the PDCCH indicates a UL transmission:

3>	if this Serving Cell is configured with uplinkHARQ-Mode:

4>	if the corresponding HARQ process is configured as HARQModeA:

5>	set HARQ-RTT-TimerUL-NTN for the corresponding HARQ process equal to drx-HARQ-RTT-TimerUL plus the latest available UE-gNB RTT value;

5>	if drx-LastTransmissionUL is configured:

6>	start the HARQ-RTT-TimerUL-NTN for the corresponding HARQ process in the first symbol after the end of the last transmission (within a bundle) of the corresponding PUSCH transmission.

5>	else:

6>	start the HARQ-RTT-TimerUL-NTN for the corresponding HARQ process in the first symbol after the end of the first transmission (within a bundle) of the corresponding PUSCH transmission.

3>	else:

4>	if drx-LastTransmissionUL is configured:

5>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the last transmission (within a bundle) of the corresponding PUSCH transmission.

4>	else:

5>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first transmission (within a bundle) of the corresponding PUSCH transmission.

3>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process.

7.1.1.5.6.3	Test description

7.1.1.5.6.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0.

System Simulator:

-	NR Cell 1 as specified in TS 38.508-1 [4] Clause 4.4.2.

-	System information combination NR-29 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used.

UE:

-	The pre-configured UE location is defined in TS 38.508-1 [4] Clause 4.5C.

7.1.1.5.6.3.2	Test procedure sequence

Table 7.1.1.5.6.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits RRCReconfiguration to configure specific DRX parameters. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits RRCReconfigurationComplete. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | In the first PDCCH occasion when the drx-onDurationTimer is running, the SS indicates the transmission of a DL MAC PDU including an RLC PDU with poll bit not set on the PDDCH. The DL MAC PDU transmitted is invalid. (Note 1) | <-- | Invalid MAC PDU | - | - |
| 4 | The UE transmits a HARQ NACK for the DL MAC PDU in Step 3. | --> | HARQ NACK | - | - |
| 5 | In the first PDCCH occasion when the drx-RetransmissionTimerDL for the MAC PDU in Step 3 is started (i.e. after expiry of HARQ-RTT-TimerDL-NTN after step 3), the SS indicates retransmission of the DL MAC PDU on the PDCCH using same HARQ process as step 3. | <-- | MAC PDU | - | - |
| 6 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 5? | --> | HARQ ACK | 1 | P |
| 7 | The SS ignores scheduling requests from UE. | - | - | - | - |
| 8 | In the next DRX cycle after Step 6, in the last PDCCH occasion when the drx-onDurationTimer is still running, the SS indicates an UL grant to the UE on the PDCCH using HARQ ID 0. | <-- | UL grant on PDCCH | - | - |
| 9 | UE loops back data received in step 5. | --> | MAC PDU | - | - |
| 10 | In the first PDCCH occasion when the drx-RetransmissionTimerUL is running, the SS indicates an UL grant to the UE on the PDCCH using HARQ ID 0 same as in step 8. | <-- | UL grant on PDCCH | - | - |
| 11 | Check: Does the UE retransmit same data in UL as in step 9? | --> | MAC PDU | 2 | P |
| 12 | SS transmits a MAC PDU containing an RLC STATUS PDU acknowledging the reception of the PDU in step 11. | <-- | MAC PDU (RLC STATUS PDU) | - | - |
| Note 1: Invalid MAC PDU is a MAC PDU that fails the CRC check. |  |  |  |  |  |

7.1.1.5.6.3.3	Specific message contents

Table 7.1.1.5.6.3.3-1: RRCReconfiguration (step 1, Table 7.1.1.5.6.3.2-1)

| Derivation Path: 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.5.6.3.3-2: CellGroupConfig (Table 7.1.1.5.6.3.3-1)

| Derivation Path: 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| cellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-BearerConfig { | 1 entry |  |  |
| RLC-Bearer-Config[1] SEQUENCE { |  |  |  |
| mac-LogicalChannelConfig SEQUENCE { |  |  |  |
| ul-SpecificParameters SEQUENCE { |  |  |  |
| allowedHARQ-mode-r17 | harqModeA |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| mac-CellGroupConfig SEQUENCE { |  |  |  |
| drx-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| drx-onDurationTimer CHOICE { |  |  |  |
| milliSeconds | ms20 |  |  |
| } |  |  |  |
| drx-InactivityTimer | ms10 |  |  |
| drx-HARQ-RTT-TimerDL | 56 |  |  |
| drx-HARQ-RTT-TimerUL | 56 |  |  |
| drx-RetransmissionTimerDL | sl8 |  |  |
| drx-RetransmissionTimerUL | sl8 |  |  |
| drx-LongCycleStartOffset CHOICE { |  |  |  |
| ms640 | 7 |  |  |
| } |  |  |  |
| shortDRX | Not present |  |  |
| drx-SlotOffset | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| schedulingRequestConfig | Not present |  |  |
| schedulingRequestConfig SEQUENCE { |  |  | NGSO |
| schedulingRequestToAddModList SEQUENCE (SIZE(1..maxNrofSR-ConfigPerCellGroup)) OF SchedulingRequestToAddMod { | 1 entry |  |  |
| SchedulingRequestToAddMod[1] SEQUENCE { |  | entry 1 |  |
| sr-TransMax | n64 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated | ServingCellConfig |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.5.6.3.3-3: ServingCellConfig (Table 7.1.1.5.6.3.3-2)

| Derivation Path: 38.508-1 [4], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP | Not present |  |  |
| initialUplinkBWP SEQUENCE { |  |  | NGSO |
| pucch-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| schedulingRequestResourceToAddModList SEQUENCE (SIZE (1..maxNrofSR-Resources)) OF SchedulingRequestResourceConfig { | 1 entry |  |  |
| SchedulingRequestResourceConfig[1] SEQUENCE { |  | entry 1 |  |
| schedulingRequestResourceId | 1 |  |  |
| schedulingRequestID | 0 |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| sl40 | 9 | 40ms for SCS 15Khz |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| pusch-ServingCellConfig CHOICE { |  |  |  |
| setup | PUSCH-ServingCellConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| pdsch-ServingCellConfig CHOICE { |  |  |  |
| setup | PDSCH-ServingCellConfig |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| NGSO | Non-geosynchronous Orbit scenario |

Table 7.1.1.5.6.3.3-4: PUSCH-ServingCellConfig (Table 7.1.1.5.6.3.3-3)

| Derivation Path: 38.508-1 [4], Table 4.6.3-121 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ServingCellConfig ::= SEQUENCE { |  |  |  |
| nrofHARQ-ProcessesForPUSCH-v1700 | n32 |  |  |
| uplinkHARQ-mode-r17 CHOICE { |  |  |  |
| setup | ‘1000 0000 0000 0000 0000 0000 0000 0000’B | HARQ ID 0 set to 1 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.5.6.3.3-5: PDSCH-ServingCellConfig (Table 7.1.1.5.6.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-102 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ServingCellConfig ::= SEQUENCE { |  |  |  |
| downlinkHARQ-FeedbackDisabled-r17 CHOICE { |  |  |  |
| setup | ‘0111 1111 1111 1111 1111 1111 1111 1111’B | HARQ ID 0 set to 0 |  |
| } |  |  |  |
| nrofHARQ-ProcessesForPDSCH-v1700 | n32 |  |  |
| } |  |  |  |

##### 7.1.1.5.7	Non-Integer DRX operation / Short cycle not configured / Parameters configured by RRC

7.1.1.5.7.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { Non-Integer Long DRX cycle is configured and drx-TimeReferenceSFN is configured and floor([(DRX_SFN_COUNTER × 10240) + (SFN × 10) + subframe number] modulo (drx-NonIntegerLongCycle)) = drx-StartOffset }

then { UE starts the OnDurationTimer and monitors the PDCCH for OnDurationTimer PDCCH-Occasions}

}

(2)

with { UE in RRC_CONNECTED state }

ensure that {

when { Non-Integer Long DRX cycle is configured and a new DL transmission is indicated on the PDCCH during Active Time }

then { UE starts or restarts the drx-InactivityTimer and monitors the PDCCH for drx-InactivityTimer PDCCH occasions starting from the next PDCCH occasion of the PDCCH occasion where the DL new transmission was indicated }

}

(3)

with { UE in RRC_CONNECTED state }

ensure that {

when { Non-Integer Long DRX cycle is configured and if a HARQ RTT Timer expires in this PDCCH  Occasion and the data in the soft buffer of the corresponding HARQ process was not successfully decoded }

then { UE starts the drx-RetransmissionTimer-DL for the corresponding HARQ process and monitors the PDCCH for drx-RetransmissionTimer consecutive PDCCH Occasion }

}

(4)

with { UE in RRC_CONNECTED state }

ensure that {

when { Non-Integer Long DRX cycle is configured and an uplink grant for a pending HARQ retransmission can occur in this PDCCH occasion }

then { UE monitors the PDCCH in this PDCCH occasion }

}

7.1.1.5.7.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.321, clauses 5.7. Unless otherwise stated these are Rel-18 requirements.

[TS 38.321, clause 5.7]

The MAC entity may be configured by RRC with a DRX functionality that controls the UE's PDCCH monitoring activity for the MAC entity's C-RNTI, CI-RNTI, CS-RNTI, INT-RNTI, SFI-RNTI, SP-CSI-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, TPC-SRS-RNTI, AI-RNTI, SL-RNTI, SL-CS-RNTI, SL Semi-Persistent Scheduling V-RNTI and cellDTRX-RNTI. When using DRX operation, the MAC entity shall also monitor PDCCH according to requirements found in other clauses of this specification. When in RRC_CONNECTED, if DRX is configured, for all the activated Serving Cells, the MAC entity may monitor the PDCCH discontinuously using the DRX operation specified in this clause; otherwise the MAC entity shall monitor the PDCCH as specified in TS 38.213 [6].

NOTE 1:	Void

RRC controls DRX operation by configuring the following parameters:

-	drx-onDurationTimer: the duration at the beginning of a DRX cycle;

-	drx-SlotOffset: the delay before starting the drx-onDurationTimer;

-	drx-InactivityTimer: the duration after the PDCCH occasion in which a PDCCH indicates a new UL, DL or SL transmission for the MAC entity;

-	drx-RetransmissionTimerDL (per DL HARQ process except for the broadcast process): the maximum duration until a DL retransmission is received;

-	drx-RetransmissionTimerUL (per UL HARQ process): the maximum duration until a grant for UL retransmission is received;

-	drx-LongCycleStartOffset: the Long DRX cycle and drx-StartOffset which defines the subframe where the Long and Short DRX cycle starts;

-	drx-NonIntegerLongCycleStartOffset (optional): the Long DRX cycle and drx-StartOffset which defines the subframe where the Long and Short DRX cycle start, when the length of the Long DRX cycle and/or the short DRX cycle is not an integer;

-	drx-ShortCycle (optional): the Short DRX cycle;

-	drx-NonIntegerShortCycle (optional): the Short DRX cycle whose length is not an integer;

-	drx-ShortCycleTimer (optional): the duration the UE shall follow the Short DRX cycle;

-	drx-HARQ-RTT-TimerDL (per DL HARQ process except for the broadcast process): the minimum duration before a DL assignment for HARQ retransmission is expected by the MAC entity;

-	drx-HARQ-RTT-TimerUL (per UL HARQ process): the minimum duration before a UL HARQ retransmission grant is expected by the MAC entity;

-	drx-RetransmissionTimerSL (per sidelink process): the maximum duration until a grant for SL retransmission is received;

-	drx-HARQ-RTT-TimerSL (per sidelink process): the minimum duration before an SL retransmission grant is expected by the MAC entity;

-	drx-LastTransmissionUL (optional): the configuration to start drx-HARQ-RTT-TimerUL after the last transmission within a bundle;

-	ps-Wakeup (optional): the configuration to start associated drx-onDurationTimer in case DCP is monitored but not detected;

-	ps-TransmitOtherPeriodicCSI (optional): the configuration to report periodic CSI that is not L1-RSRP on PUCCH during the time duration indicated by drx-onDurationTimer in case DCP is configured but associated drx-onDurationTimer is not started;

-	ps-TransmitPeriodicL1-RSRP (optional): the configuration to transmit periodic CSI that is L1-RSRP on PUCCH during the time duration indicated by drx-onDurationTimer in case DCP is configured but associated drx-onDurationTimer is not started;

-	downlinkHARQ-FeedbackDisabled (optional): the configuration to disable HARQ feedback per DL HARQ process;

-	uplinkHARQ-Mode (optional): the configuration to set HARQmodeA or HARQmodeB per UL HARQ process;

-	disableCG-RetransmissionMonitoring (optional): the configuration to disable starting drx-HARQ-RTT-TimerUL for UL transmission over a configured uplink grant;

-	drx-TimeReferenceSFN (optional): the configuration to indicate how UE initializes DRX_SFN_COUNTER.

The following UE variable is used for the DRX operation if drx-NonIntegerLongCycleStartOffset is configured:

-	DRX_SFN_COUNTER: the counter that increments when SFN changes to 0. The maximum value of this counter is at least 65535.

Serving Cells of a MAC entity may be configured by RRC in two DRX groups with separate DRX parameters. When RRC does not configure a secondary DRX group, there is only one DRX group and all Serving Cells belong to that one DRX group. When two DRX groups are configured, each Serving Cell is uniquely assigned to either of the two groups. The DRX parameters that are separately configured for each DRX group are: drx-onDurationTimer, drx-InactivityTimer. The DRX parameters that are common to the DRX groups are: drx-SlotOffset, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-LongCycleStartOffset, drx-NonIntegerLongCycleStartOffset, drx-ShortCycle (optional), drx-NonIntegerShortCycle (optional), drx-ShortCycleTimer (optional), drx-HARQ-RTT-TimerDL, and drx-HARQ-RTT-TimerUL.

When DRX is configured, the Active Time for Serving Cells in a DRX group includes the time while:

-	drx-onDurationTimer or drx-InactivityTimer configured for the DRX group is running; or

-	drx-RetransmissionTimerDL, drx-RetransmissionTimerUL or drx-RetransmissionTimerSL is running on any Serving Cell in the DRX group; or

-	ra-ContentionResolutionTimer (as described in clause 5.1.5) or msgB-ResponseWindow (as described in clause 5.1.4a) is running; or

-	a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4 or 5.22.1.5). If this Serving Cell is part of a non-terrestrial network, the Active Time is started after the Scheduling Request transmission that is performed when the SR_COUNTER is 0 for all the SR configurations with pending SR(s) plus the UE-gNB RTT; or

-	a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clauses 5.1.4 and 5.1.4a); or

-	there is an ongoing RACH-less LTM cell switch; or

-	there is an ongoing RACH-less handover in a terrestrial network.

…

When DRX is not configured and multicast DRX is configured for a G-RNTI or G-CS-RNTI, the MAC entity shall:

1>	monitor the PDCCH as specified in TS 38.213 [6];

1>	if a MAC PDU is received in a configured downlink assignment for unicast; or

…

When DRX is configured, the MAC entity shall:

1>	if a MAC PDU is received in a configured downlink assignment for unicast:

2>	if this Serving Cell is configured with downlinkHARQ-FeedbackDisabled:

…

2>	else:

3>	start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

NOTE 1a:	Void.

NOTE 1b:	Void.

2>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process;

…

1>	if a MAC PDU is transmitted in a configured uplink grant and LBT failure indication is not received from lower layers:

2>	if this Serving Cell is configured with uplinkHARQ-Mode:

…

2>	else:

3>	if disableCG-RetransmissionMonitoring is not configured for the configured uplink grant:

4>	if drx-LastTransmissionUL is configured:

5>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the last transmission (within a bundle) of the corresponding PUSCH transmission.

4>	else:

5>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first transmission (within a bundle) of the corresponding PUSCH transmission.

2>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process at the first transmission (within a bundle) of the corresponding PUSCH transmission.

…

1>	if a drx-HARQ-RTT-TimerDL expires:

2>	if the data of the corresponding HARQ process was not successfully decoded:

3>	start the drx-RetransmissionTimerDL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerDL.

…

1>	if a drx-HARQ-RTT-TimerUL expires:

2>	start the drx-RetransmissionTimerUL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerUL.

…

1>	if a DRX Command MAC CE indicated by PDCCH addressed to C-RNTI or CS-RNTI, or by a configured downlink assignment for unicast transmission or a Long DRX Command MAC CE is received:

2>	stop drx-onDurationTimer for each DRX group;

2>	stop drx-InactivityTimer for each DRX group.

1>	if drx-InactivityTimer for a DRX group expires:

2>	if the Short DRX cycle is configured:

…

2>	else:

3>	use the Long DRX cycle for this DRX group.

1>	if a DRX Command MAC CE indicated by PDCCH addressed to C-RNTI or CS-RNTI, or by a configured downlink assignment for unicast transmission is received:

2>	if the Short DRX cycle is configured:

…

2>	else:

3>	use the Long DRX cycle for each DRX group.

…

1>	if a Long DRX Command MAC CE is received:

2>	stop drx-ShortCycleTimer for each DRX group;

2>	use the Long DRX cycle for each DRX group.

1>	if the drx-NonIntegerLongCycleStartOffset is configured:

2>	increment DRX_SFN_COUNTER by 1 in the first symbol of a slot in which SFN changes to 0;

2>	if DRX is (re-)configured by RRC:

3>	if drx-TimeReferenceSFN is included in the RRC (re-)configuration which is received during the first half of a hyper frame (i.e., SFN is between 0 and 511):

4>	set DRX_SFN_COUNTER to 1.

3>	else:

4>	set DRX_SFN_COUNTER to 0.

1>	if the Short DRX cycle is used for a DRX group and the drx-NonIntegerShortCycle is not configured, and [(SFN × 10) + subframe number] modulo (drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle); or

1>	if the Short DRX cycle is used for a DRX group and the drx-NonIntegerShortCycle is configured, and floor([(DRX_SFN_COUNTER × 10240) + (SFN × 10) + subframe number - drx-StartOffset] modulo (drx-NonIntegerShortCycle)) = 0:

2>	start drx-onDurationTimer for this DRX group after drx-SlotOffset from the beginning of the subframe.

1>	if the Long DRX cycle is used for a DRX group and the drx-NonIntegerLongCycleStartOffset is not configured, and [(SFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset; or

1>	if the Long DRX cycle is used for a DRX group and the drx-NonIntegerLongCycleStartOffset is configured, and floor([(DRX_SFN_COUNTER × 10240) + (SFN × 10) + subframe number] modulo (drx-NonIntegerLongCycle)) = drx-StartOffset:

2>	if DCP monitoring is configured for the active DL BWP as specified in TS 38.213 [6], clause 10.3:

3>	if DCP indication associated with the current DRX cycle received from lower layer indicated to start drx-onDurationTimer, as specified in TS 38.213 [6]; or

3>	if all DCP occasion(s) in time domain, as specified in TS 38.213 [6], associated with the current DRX cycle occurred in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to start of the last DCP occasion, or during a measurement gap, or when the MAC entity monitors for a PDCCH transmission on the search space indicated by recoverySearchSpaceId of the SpCell identified by the C-RNTI while the ra-ResponseWindow is running (as specified in clause 5.1.4); or

3>	if ps-Wakeup is configured with value true and DCP indication associated with the current DRX cycle has not been received from lower layers:

4>	start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe.

2>	else:

3>	start drx-onDurationTimer for this DRX group after drx-SlotOffset from the beginning of the subframe.

NOTE 2:	In case of unaligned SFN across carriers in a cell group, the SFN of the SpCell is used to calculate the DRX duration.

1>	if a DRX group is in Active Time:

2>	monitor the PDCCH on the Serving Cells in this DRX group as specified in TS 38.213 [6];

2>	if the PDCCH indicates a DL transmission; or

2>	if the PDCCH indicates a one-shot HARQ feedback as specified in clause 9.1.4 of TS 38.213 [6]; or

2>	if the PDCCH indicates a retransmission of HARQ feedback as specified in clause 9.1.5 of TS 38.213 [6]:

3>	if this Serving Cell is configured with downlinkHARQ-FeedbackDisabled:

…

3>	else:

4>	start or restart the drx-HARQ-RTT-TimerDL for the corresponding HARQ process(es) whose HARQ feedback is reported in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

NOTE 3:	When HARQ feedback is postponed by PDSCH-to-HARQ_feedback timing indicating an inapplicable k1 value, as specified in TS 38.213 [6], the corresponding transmission opportunity to send the DL HARQ feedback is indicated in a later PDCCH requesting the HARQ-ACK feedback.

3>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process(es) whose HARQ feedback is reported;

3>	stop the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process;

3>	if the PDSCH-to-HARQ_feedback timing indicate an inapplicable k1 value as specified in TS 38.213 [6]:

4>	start the drx-RetransmissionTimerDL in the first symbol after the (end of the last) PDSCH transmission (within a bundle) for the corresponding HARQ process.

2>	if the PDCCH indicates a UL transmission:

3>	if this Serving Cell is configured with uplinkHARQ-Mode:

…

3>	else:

4>	if drx-LastTransmissionUL is configured:

5>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the last transmission (within a bundle) of the corresponding PUSCH transmission.

4>	else:

5>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first transmission (within a bundle) of the corresponding PUSCH transmission.

3>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process.

…

2>	if the PDCCH indicates a new transmission (DL, UL or SL) on a Serving Cell in this DRX group:

3>	start or restart drx-InactivityTimer for this DRX group in the first symbol after the end of the PDCCH reception.

NOTE 3a:	A PDCCH indicating activation of SPS, configured grant type 2, or configured sidelink grant of configured grant Type 2 is considered to indicate a new transmission.

NOTE 3b:	If the PDCCH reception includes two PDCCH candidates from corresponding search spaces, as described in clause 10.1 in 38.213, start or restart drx-InactivityTimer for this DRX group in the first symbol after the end of the PDCCH candidate that ends later in time.

2>	if a HARQ process receives downlink feedback information and acknowledgement is indicated:

3>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process.

1>	if DCP monitoring is configured for the active DL BWP as specified in TS 38.213 [6], clause 10.3; and

1>	if the current symbol n occurs within drx-onDurationTimer duration; and

1>	if drx-onDurationTimer associated with the current DRX cycle is not started as specified in this clause:

2>	if the MAC entity would not be in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause; and

2>	if allowCSI-SRS-Tx-MulticastDRX-Active is not configured, or if cfr-ConfigMulticast is not configured for any of the active BWP(s) of the Serving Cell(s), or if all multicast DRXes would not be in Active Time considering multicast assignments/DRX Command MAC CE for MBS multicast received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in Clause 5.7b and all multicast sessions are configured with multicast DRX:

3>	not transmit periodic SRS and semi-persistent SRS defined in TS 38.214 [7];

3>	not report semi-persistent CSI configured on PUSCH;

3>	not report semi-persistent CSI on PUCCH;

3>	if ps-TransmitPeriodicL1-RSRP is not configured with value true:

4>	not report periodic CSI that is L1-RSRP on PUCCH.

3>	if ps-TransmitOtherPeriodicCSI is not configured with value true:

4>	not report periodic CSI that is not L1-RSRP on PUCCH.

1>	else:

2>	in current symbol n, if a DRX group would not be in Active Time considering grants/assignments scheduled on Serving Cell(s) in this DRX group and DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause; and

2>	if allowCSI-SRS-Tx-MulticastDRX-Active is not configured, or if cfr-ConfigMulticast is not configured for any of the active BWP(s) of the Serving Cell(s), or, in current symbol n, if all multicast DRXes corresponding to the DRX group would not be in Active Time considering multicast assignments/DRX Command MAC CE for MBS multicast received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in Clause 5.7b and all multicast sessions corresponding to the DRX group are configured with multicast DRX:

3>	not transmit periodic SRS and semi-persistent SRS defined in TS 38.214 [7] in this DRX group;

3>	not report CSI on PUCCH and semi-persistent CSI configured on PUSCH in this DRX group.

2>	if CSI masking (csi-Mask) is setup by upper layers:

3>	in current symbol n, if drx-onDurationTimer of a DRX group would not be running considering grants/assignments scheduled on Serving Cell(s) in this DRX group and DRX Command MAC CE/Long DRX Command MAC CE received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause; and

3>	if allowCSI-SRS-Tx-MulticastDRX-Active is not configured, or if cfr-ConfigMulticast is not configured for any of the active BWP(s) of the Serving Cell(s), or, in current symbol n, if drx-onDurationTimerPTM(s) of all multicast DRXes corresponding to the DRX group would not be running considering DRX Command MAC CE for MBS multicast received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in Clause 5.7b and all multicast sessions corresponding to the DRX group are configured with multicast DRX:

4>	not report CSI on PUCCH in this DRX group.

NOTE 4:	If a UE multiplexes a CSI configured on PUCCH with other overlapping UCI(s) according to the procedure specified in TS 38.213 [6] clause 9.2.5 and this CSI multiplexed with other UCI(s) would be reported on a PUCCH resource either outside DRX Active Time of the DRX group in which this PUCCH is configured or outside the on-duration period of the DRX group in which this PUCCH is configured if CSI masking is setup by upper layers, it is up to UE implementation whether to report this CSI multiplexed with other UCI(s).

The MAC entity shall ensure no rounding error is generated when performing the modulus operation with drx-NonIntegerShortCycle or drx-NonIntegerLongCycle as the divisor.

Regardless of whether the MAC entity is monitoring PDCCH or not on the Serving Cells in a DRX group, the MAC entity transmits HARQ feedback, aperiodic CSI on PUSCH, and aperiodic SRS defined in TS 38.214 [7] on the Serving Cells in the DRX group when such is expected.

The MAC entity needs not to monitor the PDCCH if it is not a complete PDCCH occasion (e.g. the Active Time starts or ends in the middle of a PDCCH occasion).

7.1.1.5.7.3	Test description

7.1.1.5.7.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that set to return no data in uplink.

7.1.1.5.7.3.2	Test procedure sequence

Table 7.1.1.5.7.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits RRCReconfiguration to configure specific DRX parameters. | <-- | - | - | - |
| 2 | The UE transmits RRCReconfigurationComplete. | --> | - | - | - |
| 3 | On the first PDCCH occasion while drx-onDurationTimer is running (first DRX cycle after RRCReconfigurationComplete), the SS indicates the transmission of a DL MAC PDU. | <-- | MAC PDU | - | - |
| 4 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 3? | --> | HARQ ACK | 1 | P |
| 5 | After drx-InactivityTimer period of the DL MAC PDU transmission indication on the PDCCH in Step 3, in the last PDCCH occasion of the ongoing drx-onDurationTimer period (second DRX cycle after RRCReconfigurationComplete), the SS indicates the transmission of a DL MAC PDU. | <-- | MAC PDU | - | - |
| 6 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 5? | --> | HARQ ACK | 1 | P |
| 7 | On the PDCCH occasion during drx-InactivityTimer period of the DL MAC PDU transmission indication on the PDCCH in Step 5, the SS indicates transmission of a DL MAC PDU. | <-- | MAC PDU | - | - |
| 8 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 7? | --> | HARQ ACK | 2 | P |
| 9 | After drx-InactivityTimer period of the DL MAC PDU transmission indication on the PDCCH in Step 7, in the last PDCCH occasion of the ongoing drx-onDurationTimer period (third DRX cycle after RRCReconfigurationComplete), the SS indicates the transmission of a DL MAC PDU. The DL MAC PDU transmitted is invalid. | <-- | Invalid MAC PDU | - | - |
| 10 | Check: Does the UE transmit a HARQ NACK for the DL MAC PDU in Step 9? | --> | HARQ NACK | 1 | P |
| 11 | After expiry of drx-HARQ-RTT-TimerDL for the failed HARQ in step 10, in the first PDCCH occasion of the ongoing drx-RetransmissionTimerDL period, the SS indicates the transmission of a DL MAC PDU. | <-- | MAC PDU | - | - |
| 12 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 11? | --> | HARQ ACK | 3 | P |
| - | Wait for 10 DRX cycles. | - | - | - | - |
| 13 | On the last PDCCH occasion of the ongoing drx-onDurationTimer period (14th DRX cycle after RRCReconfigurationComplete), the SS indicates the transmission of a DL MAC PDU. The DL MAC PDU transmitted is invalid. | <-- | Invalid MAC PDU | - | - |
| 14 | Check: Does the UE transmit a HARQ NACK for the DL MAC PDU in Step 13? | --> | HARQ NACK | 1 | P |
| 15 | After expiry of drx-HARQ-RTT-TimerDL for the failed HARQ in step 14, in the last PDCCH occasion of the ongoing drx-RetransmissionTimerDL period, the SS indicates the transmission of a DL MAC PDU. | <-- | MAC PDU | - | - |
| 16 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 15? | --> | HARQ ACK | 3 | P |
| 17 | The SS is configured for Uplink Grant Allocation Type [0]. After drx-InactivityTimer period of the DL MAC PDU transmission indication on the PDCCH in Step 15, in the last PDCCH occasion of the ongoing drx-onDurationTimer period (15th DRX cycle after RRCReconfigurationComplete), the SS indicates an UL grant to the UE on the PDCCH. | <-- | UL grant on PDCCH | - | - |
| 18 | Check: Does the UE transmit a Buffer Status Report on the UL indicating an empty buffer? | --> | Buffer Status Report MAC control element | 1 | P |
| 19 | After expiry of drx-HARQ-RTT-TimerUL for the failed HARQ in step 17, in the last PDCCH occasion of the ongoing drx-RetransmissionTimer-UL period, the SS indicates the transmission of a DL MAC PDU. | <-- | MAC PDU | - | - |
| 20 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 19? | --> | HARQ ACK | 4 | P |
| 21 | On the last PDCCH occasion while drx-onDurationTimer is running (16th DRX cycle after RRCReconfigurationComplete), the SS indicates the transmission of a DL MAC PDU. | <-- | MAC PDU | - | - |
| 22 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 21? | --> | HARQ ACK | 1 | P |

7.1.1.5.7.3.3	Specific message contents

Table 7.1.1.5.7.3.3-1: RRCReconfiguration (step 1, Table 7.1.1.5.7.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-13

Table 7.1.1.5.7.3.3-2: CellGroupConfig (Table 7.1.1.5.7.3.3-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.3-19

Table 7.1.1.5.7.3.3-3: MAC-CellGroupConfig (Table 7.1.1.5.7.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-68 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MAC-CellGroupConfig ::= SEQUENCE { |  |  |  |
| drx-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| drx-onDurationTimer | Ms2 |  |  |
| drx-InactivityTimer | Ms1 |  |  |
| drx-HARQ-RTT-TimerDL | 56 | Number of slots=4 due to number of symbol per slot=14 | µ = 0,1,2,3,4 ( 2 with normal CP) |
|  | 48 | Number of slots=4 due to number of symbol per slot=12 | µ = 2 with extended CP |
| drx-HARQ-RTT-TimerUL | 56 | Number of slots=4 due to number of symbol per slot=14 | µ = 0,1,2,3,4 ( 2 with normal CP) |
|  | 48 | Number of slots=4 due to number of symbol per slot=12 | µ = 2 with extended CP |
| drx-RetransmissionTimerDL | sl8 |  |  |
| drx-RetransmissionTimerUL | sl8 |  |  |
| drx-LongCycleStartOffset CHOICE { | Not present |  |  |
| ms640 | 7 |  |  |
| } |  |  |  |
| shortDRX | Not present |  |  |
| drx-SlotOffset | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| drx-ConfigExt2-v1800 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| drx-NonIntegerLongCycleStartOffset-r18 CHOICE { |  |  |  |
| ms25over3 | 0 |  |  |
| } |  |  |  |
| shortDRX-r18 | Not present |  |  |
| drx-TimeReferenceSFN-r18 | sfn512 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 7.1.1.5.8	Non-Integer DRX operation / Short cycle configured / Parameters configured by RRC

7.1.1.5.8.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { Non-Integer Short DRX cycle is configured and drx-TimeReferenceSFN is configured and drx-SlotOffset is configured and floor([(DRX_SFN_COUNTER × 10240) + (SFN × 10) + subframe number] modulo (drx-NonIntegerShortCycle)) = floor([(drx-TimeReferenceSFN × 10) + drx-StartOffset] modulo (drx-NonIntegerShortCycle)) }

then { UE starts the OnDurationTimer after drx-SlotOffset and monitors the PDCCH for OnDurationTimer PDCCH-subframes }

}

(2)

with { UE in RRC_CONNECTED state }

ensure that {

when { drxShortCycleTimer is expired and floor([(DRX_SFN_COUNTER × 10240) + (SFN × 10) + subframe number] modulo (drx-NonIntegerLongCycle)) = floor([(drx-TimeReferenceSFN × 10) + drx-StartOffset] modulo (drx-NonIntegerLongCycle)) }

then { UE starts the OnDurationTimer after drx-SlotOffset and monitors the PDCCH for OnDurationTimer PDCCH-subframes }

}

7.1.1.5.8.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.321, clauses 5.7. Unless otherwise stated these are Rel-18 requirements.

[TS 38.321, clause 5.7]

The MAC entity may be configured by RRC with a DRX functionality that controls the UE's PDCCH monitoring activity for the MAC entity's C-RNTI, CI-RNTI, CS-RNTI, INT-RNTI, SFI-RNTI, SP-CSI-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, TPC-SRS-RNTI, AI-RNTI, SL-RNTI, SL-CS-RNTI, SL Semi-Persistent Scheduling V-RNTI and cellDTRX-RNTI. When using DRX operation, the MAC entity shall also monitor PDCCH according to requirements found in other clauses of this specification. When in RRC_CONNECTED, if DRX is configured, for all the activated Serving Cells, the MAC entity may monitor the PDCCH discontinuously using the DRX operation specified in this clause; otherwise the MAC entity shall monitor the PDCCH as specified in TS 38.213 [6].

NOTE 1:	Void

RRC controls DRX operation by configuring the following parameters:

-	drx-onDurationTimer: the duration at the beginning of a DRX cycle;

-	drx-SlotOffset: the delay before starting the drx-onDurationTimer;

-	drx-InactivityTimer: the duration after the PDCCH occasion in which a PDCCH indicates a new UL, DL or SL transmission for the MAC entity;

-	drx-RetransmissionTimerDL (per DL HARQ process except for the broadcast process): the maximum duration until a DL retransmission is received;

-	drx-RetransmissionTimerUL (per UL HARQ process): the maximum duration until a grant for UL retransmission is received;

-	drx-LongCycleStartOffset: the Long DRX cycle and drx-StartOffset which defines the subframe where the Long and Short DRX cycle starts;

-	drx-NonIntegerLongCycleStartOffset (optional): the Long DRX cycle and drx-StartOffset which defines the subframe where the Long and Short DRX cycle start, when the length of the Long DRX cycle and/or the short DRX cycle is not an integer;

-	drx-ShortCycle (optional): the Short DRX cycle;

-	drx-NonIntegerShortCycle (optional): the Short DRX cycle whose length is not an integer;

-	drx-ShortCycleTimer (optional): the duration the UE shall follow the Short DRX cycle;

-	drx-HARQ-RTT-TimerDL (per DL HARQ process except for the broadcast process): the minimum duration before a DL assignment for HARQ retransmission is expected by the MAC entity;

-	drx-HARQ-RTT-TimerUL (per UL HARQ process): the minimum duration before a UL HARQ retransmission grant is expected by the MAC entity;

-	drx-RetransmissionTimerSL (per sidelink process): the maximum duration until a grant for SL retransmission is received;

-	drx-HARQ-RTT-TimerSL (per sidelink process): the minimum duration before an SL retransmission grant is expected by the MAC entity;

-	drx-LastTransmissionUL (optional): the configuration to start drx-HARQ-RTT-TimerUL after the last transmission within a bundle;

-	ps-Wakeup (optional): the configuration to start associated drx-onDurationTimer in case DCP is monitored but not detected;

-	ps-TransmitOtherPeriodicCSI (optional): the configuration to report periodic CSI that is not L1-RSRP on PUCCH during the time duration indicated by drx-onDurationTimer in case DCP is configured but associated drx-onDurationTimer is not started;

-	ps-TransmitPeriodicL1-RSRP (optional): the configuration to transmit periodic CSI that is L1-RSRP on PUCCH during the time duration indicated by drx-onDurationTimer in case DCP is configured but associated drx-onDurationTimer is not started;

-	downlinkHARQ-FeedbackDisabled (optional): the configuration to disable HARQ feedback per DL HARQ process;

-	uplinkHARQ-Mode (optional): the configuration to set HARQmodeA or HARQmodeB per UL HARQ process;

-	disableCG-RetransmissionMonitoring (optional): the configuration to disable starting drx-HARQ-RTT-TimerUL for UL transmission over a configured uplink grant;

-	drx-TimeReferenceSFN (optional): the configuration to indicate how UE initializes DRX_SFN_COUNTER.

The following UE variable is used for the DRX operation if drx-NonIntegerLongCycleStartOffset is configured:

-	DRX_SFN_COUNTER: the counter that increments when SFN changes to 0. The maximum value of this counter is at least 65535.

Serving Cells of a MAC entity may be configured by RRC in two DRX groups with separate DRX parameters. When RRC does not configure a secondary DRX group, there is only one DRX group and all Serving Cells belong to that one DRX group. When two DRX groups are configured, each Serving Cell is uniquely assigned to either of the two groups. The DRX parameters that are separately configured for each DRX group are: drx-onDurationTimer, drx-InactivityTimer. The DRX parameters that are common to the DRX groups are: drx-SlotOffset, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-LongCycleStartOffset, drx-NonIntegerLongCycleStartOffset, drx-ShortCycle (optional), drx-NonIntegerShortCycle (optional), drx-ShortCycleTimer (optional), drx-HARQ-RTT-TimerDL, and drx-HARQ-RTT-TimerUL.

When DRX is configured, the Active Time for Serving Cells in a DRX group includes the time while:

-	drx-onDurationTimer or drx-InactivityTimer configured for the DRX group is running; or

-	drx-RetransmissionTimerDL, drx-RetransmissionTimerUL or drx-RetransmissionTimerSL is running on any Serving Cell in the DRX group; or

-	ra-ContentionResolutionTimer (as described in clause 5.1.5) or msgB-ResponseWindow (as described in clause 5.1.4a) is running; or

-	a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4 or 5.22.1.5). If this Serving Cell is part of a non-terrestrial network, the Active Time is started after the Scheduling Request transmission that is performed when the SR_COUNTER is 0 for all the SR configurations with pending SR(s) plus the UE-gNB RTT; or

-	a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clauses 5.1.4 and 5.1.4a); or

-	there is an ongoing RACH-less LTM cell switch; or

-	there is an ongoing RACH-less handover in a terrestrial network.

The following MAC timers are used for DRX operation in a non-terrestrial network:

-	HARQ-RTT-TimerDL-NTN (per DL HARQ process configured with HARQ feedback enabled): the minimum duration before a DL assignment for HARQ retransmission is expected by the MAC entity;

-	HARQ-RTT-TimerUL-NTN (per UL HARQ process configured with HARQModeA): the minimum duration before a UL HARQ retransmission grant is expected by the MAC entity.

When DRX is not configured and multicast DRX is configured for a G-RNTI or G-CS-RNTI, the MAC entity shall:

1>	monitor the PDCCH as specified in TS 38.213 [6];

1>	if a MAC PDU is received in a configured downlink assignment for unicast; or

1>	if the PDCCH indicates a DL unicast transmission:

2>	stop the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process.

When DRX is configured, the MAC entity shall:

1>	if a MAC PDU is received in a configured downlink assignment for unicast:

2>	if this Serving Cell is configured with downlinkHARQ-FeedbackDisabled:

…

2>	else:

3>	start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

NOTE 1a:	Void.

NOTE 1b:	Void.

2>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process;

2>	stop the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process.

1>	if a MAC PDU is transmitted in a configured uplink grant and LBT failure indication is not received from lower layers:

2>	if this Serving Cell is configured with uplinkHARQ-Mode:

…

2>	else:

3>	if disableCG-RetransmissionMonitoring is not configured for the configured uplink grant:

4>	if drx-LastTransmissionUL is configured:

5>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the last transmission (within a bundle) of the corresponding PUSCH transmission.

4>	else:

5>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first transmission (within a bundle) of the corresponding PUSCH transmission.

2>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process at the first transmission (within a bundle) of the corresponding PUSCH transmission.

…

1>	if a drx-HARQ-RTT-TimerDL expires:

2>	if the data of the corresponding HARQ process was not successfully decoded:

3>	start the drx-RetransmissionTimerDL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerDL.

…

1>	if a drx-HARQ-RTT-TimerUL expires:

2>	start the drx-RetransmissionTimerUL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerUL.

…

1>	if a DRX Command MAC CE indicated by PDCCH addressed to C-RNTI or CS-RNTI, or by a configured downlink assignment for unicast transmission or a Long DRX Command MAC CE is received:

2>	stop drx-onDurationTimer for each DRX group;

2>	stop drx-InactivityTimer for each DRX group.

1>	if drx-InactivityTimer for a DRX group expires:

2>	if the Short DRX cycle is configured:

3>	start or restart drx-ShortCycleTimer for this DRX group in the first symbol after the expiry of drx-InactivityTimer;

3>	use the Short DRX cycle for this DRX group.

2>	else:

3>	use the Long DRX cycle for this DRX group.

1>	if a DRX Command MAC CE indicated by PDCCH addressed to C-RNTI or CS-RNTI, or by a configured downlink assignment for unicast transmission is received:

2>	if the Short DRX cycle is configured:

3>	start or restart drx-ShortCycleTimer for each DRX group in the first symbol after the end of DRX Command MAC CE reception;

3>	use the Short DRX cycle for each DRX group.

2>	else:

3>	use the Long DRX cycle for each DRX group.

1>	if drx-ShortCycleTimer for a DRX group expires:

2>	use the Long DRX cycle for this DRX group.

1>	if a Long DRX Command MAC CE is received:

2>	stop drx-ShortCycleTimer for each DRX group;

2>	use the Long DRX cycle for each DRX group.

1>	if the drx-NonIntegerLongCycleStartOffset is configured:

2>	increment DRX_SFN_COUNTER by 1 in the first symbol of a slot in which SFN changes to 0;

2>	if DRX is (re-)configured by RRC:

3>	if drx-TimeReferenceSFN is included in the RRC (re-)configuration which is received during the first half of a hyper frame (i.e., SFN is between 0 and 511):

4>	set DRX_SFN_COUNTER to 1.

3>	else:

4>	set DRX_SFN_COUNTER to 0.

1>	if the Short DRX cycle is used for a DRX group and the drx-NonIntegerShortCycle is not configured, and [(SFN × 10) + subframe number] modulo (drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle); or

1>	if the Short DRX cycle is used for a DRX group and the drx-NonIntegerShortCycle is configured, and floor([(DRX_SFN_COUNTER × 10240) + (SFN × 10) + subframe number - drx-StartOffset] modulo (drx-NonIntegerShortCycle)) = 0:

2>	start drx-onDurationTimer for this DRX group after drx-SlotOffset from the beginning of the subframe.

1>	if the Long DRX cycle is used for a DRX group and the drx-NonIntegerLongCycleStartOffset is not configured, and [(SFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset; or

1>	if the Long DRX cycle is used for a DRX group and the drx-NonIntegerLongCycleStartOffset is configured, and floor([(DRX_SFN_COUNTER × 10240) + (SFN × 10) + subframe number] modulo (drx-NonIntegerLongCycle)) = drx-StartOffset:

2>	if DCP monitoring is configured for the active DL BWP as specified in TS 38.213 [6], clause 10.3:

3>	if DCP indication associated with the current DRX cycle received from lower layer indicated to start drx-onDurationTimer, as specified in TS 38.213 [6]; or

3>	if all DCP occasion(s) in time domain, as specified in TS 38.213 [6], associated with the current DRX cycle occurred in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to start of the last DCP occasion, or during a measurement gap, or when the MAC entity monitors for a PDCCH transmission on the search space indicated by recoverySearchSpaceId of the SpCell identified by the C-RNTI while the ra-ResponseWindow is running (as specified in clause 5.1.4); or

3>	if ps-Wakeup is configured with value true and DCP indication associated with the current DRX cycle has not been received from lower layers:

4>	start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe.

2>	else:

3>	start drx-onDurationTimer for this DRX group after drx-SlotOffset from the beginning of the subframe.

NOTE 2:	In case of unaligned SFN across carriers in a cell group, the SFN of the SpCell is used to calculate the DRX duration.

1>	if a DRX group is in Active Time:

2>	monitor the PDCCH on the Serving Cells in this DRX group as specified in TS 38.213 [6];

2>	if the PDCCH indicates a DL transmission; or

2>	if the PDCCH indicates a one-shot HARQ feedback as specified in clause 9.1.4 of TS 38.213 [6]; or

2>	if the PDCCH indicates a retransmission of HARQ feedback as specified in clause 9.1.5 of TS 38.213 [6]:

3>	if this Serving Cell is configured with downlinkHARQ-FeedbackDisabled:

4>	if the corresponding HARQ process is configured with HARQ feedback enabled:

5>	set HARQ-RTT-TimerDL-NTN for the corresponding HARQ process equal to drx-HARQ-RTT-TimerDL plus the latest available UE-gNB RTT value;

5>	start the HARQ-RTT-TimerDL-NTN for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

3>	else:

4>	start or restart the drx-HARQ-RTT-TimerDL for the corresponding HARQ process(es) whose HARQ feedback is reported in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

NOTE 3:	When HARQ feedback is postponed by PDSCH-to-HARQ_feedback timing indicating an inapplicable k1 value, as specified in TS 38.213 [6], the corresponding transmission opportunity to send the DL HARQ feedback is indicated in a later PDCCH requesting the HARQ-ACK feedback.

3>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process(es) whose HARQ feedback is reported;

3>	stop the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process;

3>	if the PDSCH-to-HARQ_feedback timing indicate an inapplicable k1 value as specified in TS 38.213 [6]:

4>	start the drx-RetransmissionTimerDL in the first symbol after the (end of the last) PDSCH transmission (within a bundle) for the corresponding HARQ process.

2>	if the PDCCH indicates a UL transmission:

3>	if this Serving Cell is configured with uplinkHARQ-Mode:

4>	if the corresponding HARQ process is configured as HARQModeA:

5>	set HARQ-RTT-TimerUL-NTN for the corresponding HARQ process equal to drx-HARQ-RTT-TimerUL plus the latest available UE-gNB RTT value;

5>	if drx-LastTransmissionUL is configured:

6>	start the HARQ-RTT-TimerUL-NTN for the corresponding HARQ process in the first symbol after the end of the last transmission (within a bundle) of the corresponding PUSCH transmission.

5>	else:

6>	start the HARQ-RTT-TimerUL-NTN for the corresponding HARQ process in the first symbol after the end of the first transmission (within a bundle) of the corresponding PUSCH transmission.

3>	else:

4>	if drx-LastTransmissionUL is configured:

5>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the last transmission (within a bundle) of the corresponding PUSCH transmission.

4>	else:

5>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first transmission (within a bundle) of the corresponding PUSCH transmission.

3>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process.

2>	if the PDCCH indicates an SL transmission:

3>	if the PUCCH resource is configured:

4>	start the drx-HARQ-RTT-TimerSL for the corresponding HARQ process in the first symbol after the end of the corresponding PUCCH transmission carrying the SL HARQ feedback; or

4>	start the drx-HARQ-RTT-TimerSL for the corresponding HARQ process in the first symbol after the end of the corresponding PUCCH resource for the SL HARQ feedback when the PUCCH is not transmitted;

4>	stop the drx-RetransmissionTimerSL for the corresponding HARQ process.

3>	else:

4>	start the drx-HARQ-RTT-TimerSL for the corresponding HARQ process at the first symbol after end of PDCCH occasion;

4>	stop the drx-RetransmissionTimerSL for the corresponding HARQ process.

2>	if the PDCCH indicates a new transmission (DL, UL or SL) on a Serving Cell in this DRX group:

3>	start or restart drx-InactivityTimer for this DRX group in the first symbol after the end of the PDCCH reception.

NOTE 3a:	A PDCCH indicating activation of SPS, configured grant type 2, or configured sidelink grant of configured grant Type 2 is considered to indicate a new transmission.

NOTE 3b:	If the PDCCH reception includes two PDCCH candidates from corresponding search spaces, as described in clause 10.1 in 38.213, start or restart drx-InactivityTimer for this DRX group in the first symbol after the end of the PDCCH candidate that ends later in time.

2>	if a HARQ process receives downlink feedback information and acknowledgement is indicated:

3>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process.

1>	if DCP monitoring is configured for the active DL BWP as specified in TS 38.213 [6], clause 10.3; and

1>	if the current symbol n occurs within drx-onDurationTimer duration; and

1>	if drx-onDurationTimer associated with the current DRX cycle is not started as specified in this clause:

2>	if the MAC entity would not be in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause; and

2>	if allowCSI-SRS-Tx-MulticastDRX-Active is not configured, or if cfr-ConfigMulticast is not configured for any of the active BWP(s) of the Serving Cell(s), or if all multicast DRXes would not be in Active Time considering multicast assignments/DRX Command MAC CE for MBS multicast received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in Clause 5.7b and all multicast sessions are configured with multicast DRX:

3>	not transmit periodic SRS and semi-persistent SRS defined in TS 38.214 [7];

3>	not report semi-persistent CSI configured on PUSCH;

3>	not report semi-persistent CSI on PUCCH;

3>	if ps-TransmitPeriodicL1-RSRP is not configured with value true:

4>	not report periodic CSI that is L1-RSRP on PUCCH.

3>	if ps-TransmitOtherPeriodicCSI is not configured with value true:

4>	not report periodic CSI that is not L1-RSRP on PUCCH.

1>	else:

2>	in current symbol n, if a DRX group would not be in Active Time considering grants/assignments scheduled on Serving Cell(s) in this DRX group and DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause; and

2>	if allowCSI-SRS-Tx-MulticastDRX-Active is not configured, or if cfr-ConfigMulticast is not configured for any of the active BWP(s) of the Serving Cell(s), or, in current symbol n, if all multicast DRXes corresponding to the DRX group would not be in Active Time considering multicast assignments/DRX Command MAC CE for MBS multicast received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in Clause 5.7b and all multicast sessions corresponding to the DRX group are configured with multicast DRX:

3>	not transmit periodic SRS and semi-persistent SRS defined in TS 38.214 [7] in this DRX group;

3>	not report CSI on PUCCH and semi-persistent CSI configured on PUSCH in this DRX group.

2>	if CSI masking (csi-Mask) is setup by upper layers:

3>	in current symbol n, if drx-onDurationTimer of a DRX group would not be running considering grants/assignments scheduled on Serving Cell(s) in this DRX group and DRX Command MAC CE/Long DRX Command MAC CE received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause; and

3>	if allowCSI-SRS-Tx-MulticastDRX-Active is not configured, or if cfr-ConfigMulticast is not configured for any of the active BWP(s) of the Serving Cell(s), or, in current symbol n, if drx-onDurationTimerPTM(s) of all multicast DRXes corresponding to the DRX group would not be running considering DRX Command MAC CE for MBS multicast received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in Clause 5.7b and all multicast sessions corresponding to the DRX group are configured with multicast DRX:

4>	not report CSI on PUCCH in this DRX group.

NOTE 4:	If a UE multiplexes a CSI configured on PUCCH with other overlapping UCI(s) according to the procedure specified in TS 38.213 [6] clause 9.2.5 and this CSI multiplexed with other UCI(s) would be reported on a PUCCH resource either outside DRX Active Time of the DRX group in which this PUCCH is configured or outside the on-duration period of the DRX group in which this PUCCH is configured if CSI masking is setup by upper layers, it is up to UE implementation whether to report this CSI multiplexed with other UCI(s).

The MAC entity shall ensure no rounding error is generated when performing the modulus operation with drx-NonIntegerShortCycle or drx-NonIntegerLongCycle as the divisor.

Regardless of whether the MAC entity is monitoring PDCCH or not on the Serving Cells in a DRX group, the MAC entity transmits HARQ feedback, aperiodic CSI on PUSCH, and aperiodic SRS defined in TS 38.214 [7] on the Serving Cells in the DRX group when such is expected.

The MAC entity needs not to monitor the PDCCH if it is not a complete PDCCH occasion (e.g. the Active Time starts or ends in the middle of a PDCCH occasion).

7.1.1.5.8.3	Test description

7.1.1.5.8.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that set to return no data in uplink.

7.1.1.5.8.3.2	Test procedure sequence

Table 7.1.1.5.8.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | SS transmits NR RRCReconfiguration message to configure specific DRX parameters for SpCell. | <-- | - | - | - |
| 2 | The UE transmitNR RRCReconfigurationComplete messages. | --> | - | - | - |
| 3 | In the first PDCCH occasion, after the drx-SlotOffset when the drx-onDurationTimer is running (first long DRX cycle after step 1), the SS indicates the transmission of a DL MAC PDU on the PDCCH. (Note 3)(Note 4)(Note 5) | <-- | MAC PDU | - | - |
| 4 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 3? | --> | HARQ ACK | 1 | P |
| 5 | At least drx-InactivityTimer after the transmission of the MAC PDU in Step 3 has been indicated (This means the next short DRX cycle or later after Step 3) in the last PDCCH occasion while the drx-onDurationTimer is still running according to floor([(DRX_SFN_COUNTER × 10240) + (SFN × 10) + subframe number] modulo (drx-NonIntegerShortCycle)) = floor([(drx-TimeReferenceSFN × 10) + drx-StartOffset] modulo (drx-NonIntegerShortCycle)), the SS indicates the transmission a DL MAC PDU on the PDDCH. | <-- | MAC PDU | - | - |
| 6 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 5? | --> | HARQ ACK | 1 | P |
| 7 | Wait 14 long cycle PDDCH occasions. | - | - | - | - |
| 8 | In the first PDCCH occasion after the drx-SlotOffset when the drx-onDurationTimer of drx-NonIntegerLongCycle is running (16th long DRX cycle after step 1), the SS indicates the transmission of a DL MAC PDU on the PDCCH. (Note 3)(Note 4)(Note 5) | <-- | MAC PDU | - | - |
| 9 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 8? | --> | HARQ ACK | 1 | P |
| 10 | At least drx-InactivityTimer after the transmission of the MAC PDU in Step 8 has been indicated (This means the next short DRX cycle or later after Step 8) in the last PDCCH occasion while the drx-onDurationTimer is still running according to floor([(DRX_SFN_COUNTER × 10240) + (SFN × 10) + subframe number] modulo (drx-NonIntegerShortCycle)) = floor([(drx-TimeReferenceSFN × 10) + drx-StartOffset] modulo (drx-NonIntegerShortCycle)), the SS indicates the transmission a DL MAC PDU on the PDDCH. | <-- | MAC PDU | - | - |
| 11 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 10? | --> | HARQ ACK | 1 | P |
| 12 | SS waits for drx-ShortCycleTimer to expire. | - | - | - | - |
| 13 | In the first PDCCH occasion after the drx-SlotOffset when the drx-onDurationTimer of drx-NonIntegerLongCycle is running (17th long DRX cycle after step 1), the SS indicates the transmission of a DL MAC PDU on the PDCCH. (Note 3)(Note 4)(Note 5) | <-- | MAC PDU | - | - |
| 14 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 9? | --> | HARQ ACK | 2 | P |
| Note 1: Void. Note 2: Void. Note 3: The drx-InactivityTimer is started in the first symbol after the end of the PDCCH reception where DL new transmission is indicated. Note 4: When the drx-InactivityTimer expires, UE starts drx-ShortCycleTimer in the first symbol after the expiry of drx-InactivityTimer. Note 5: The SS assumes that the UE starts in long DRX after configuration. |  |  |  |  |  |

7.1.1.5.8.3.3	Specific message contents

Table 7.1.1.5.8.3.3-1: RRCReconfiguration (step 1, Table 7.1.1.5.8.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-13

Table 7.1.1.5.8.3.3-2: CellGroupConfig (Table 7.1.1.5.8.3.3-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.3-19

Table 7.1.1.5.8.3.3-3: MAC-CellGroupConfig (Table 7.1.1.5.8.3.3-2)

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MAC-CellGroupConfig ::= SEQUENCE { |  |  |  |
| drx-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| drx-onDurationTimer | ms5 |  |  |
| drx-InactivityTimer | ms2 |  |  |
| drx-HARQ-RTT-TimerDL | 56 | Number of slots=4 due to number of symbol per slot=14 | µ = 0,1,2,3,4 ( 2 with normal CP) |
|  | 48 | Number of slots=4 due to number of symbol per slot=12 | µ = 2 with external CP |
| drx-HARQ-RTT-TimerUL | 56 | Number of slots=4 due to number of symbol per slot=14 | µ = 0,1,2,3,4 ( 2 with normal CP) |
|  | 48 | Number of slots=4 due to number of symbol per slot=12 | µ = 2 with external CP |
| drx-RetransmissionTimerDL | sl8 |  |  |
| drx-RetransmissionTimerUL | sl8 |  |  |
| drx-LongCycleStartOffset | Not present |  |  |
| shortDRX | Not present |  |  |
| drx-SlotOffset | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| drx-ConfigExt-v1800 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| drx-NonIntegerLongCycleStartOffset-r18 CHOICE { |  |  |  |
| ms25over3 | 0 |  |  |
| } |  |  |  |
| shortDRX-r18 SEQUENCE { |  |  |  |
| drx-NonIntegerShortCycle-r18 | ms25over6 |  |  |
| drx-ShortCycleTimer-r18 | 1 |  |  |
| } |  |  |  |
| drx-TimeReferenceSFN-r18 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 7.1.1.5.9	Non-Integer DRX operation / Short cycle not configured / DRX_SFN_COUNTER reset after RRC re-configuration

7.1.1.5.9.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { Non-Integer Long DRX cycle is configured and RRC (re-)configuration message is received during the first half of a hyper frame and drx-TimeReferenceSFN is included }

then { UE sets DRX_SFN_COUNTER to 1 in the first symbol of the slot immediately after the first PUSCH transmission which contains the RRCReconfigurationComplete message of the RRC (re-) configuration as specified in TS 38.331 }

}

(2)

with { UE in RRC_CONNECTED state }

ensure that {

when { Non-Integer Long DRX cycle is configured and RRC (re-)configuration message is received during the first half of a hyper frame and drx-TimeReferenceSFN is not included) }

then { UE sets DRX_SFN_COUNTER to 0 in the first symbol of the slot immediately after the first PUSCH transmission which contains the RRCReconfigurationComplete message of the RRC (re-) configuration as specified in TS 38.331 }

}

7.1.1.5.9.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.321, clauses 5.7. Unless otherwise stated these are Rel-18 requirements.

[TS 38.321, clause 5.7]

The MAC entity may be configured by RRC with a DRX functionality that controls the UE's PDCCH monitoring activity for the MAC entity's C-RNTI, CI-RNTI, CS-RNTI, INT-RNTI, SFI-RNTI, SP-CSI-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, TPC-SRS-RNTI, AI-RNTI, SL-RNTI, SL-CS-RNTI, SL Semi-Persistent Scheduling V-RNTI and cellDTRX-RNTI. When using DRX operation, the MAC entity shall also monitor PDCCH according to requirements found in other clauses of this specification. When in RRC_CONNECTED, if DRX is configured, for all the activated Serving Cells, the MAC entity may monitor the PDCCH discontinuously using the DRX operation specified in this clause; otherwise the MAC entity shall monitor the PDCCH as specified in TS 38.213 [6].

NOTE 1:	Void

RRC controls DRX operation by configuring the following parameters:

-	drx-onDurationTimer: the duration at the beginning of a DRX cycle;

-	drx-SlotOffset: the delay before starting the drx-onDurationTimer;

-	drx-InactivityTimer: the duration after the PDCCH occasion in which a PDCCH indicates a new UL, DL or SL transmission for the MAC entity;

-	drx-RetransmissionTimerDL (per DL HARQ process except for the broadcast process): the maximum duration until a DL retransmission is received;

-	drx-RetransmissionTimerUL (per UL HARQ process): the maximum duration until a grant for UL retransmission is received;

-	drx-LongCycleStartOffset: the Long DRX cycle and drx-StartOffset which defines the subframe where the Long and Short DRX cycle starts;

-	drx-NonIntegerLongCycleStartOffset (optional): the Long DRX cycle and drx-StartOffset which defines the subframe where the Long and Short DRX cycle start, when the length of the Long DRX cycle and/or the short DRX cycle is not an integer;

-	drx-ShortCycle (optional): the Short DRX cycle;

-	drx-NonIntegerShortCycle (optional): the Short DRX cycle whose length is not an integer;

-	drx-ShortCycleTimer (optional): the duration the UE shall follow the Short DRX cycle;

-	drx-HARQ-RTT-TimerDL (per DL HARQ process except for the broadcast process): the minimum duration before a DL assignment for HARQ retransmission is expected by the MAC entity;

-	drx-HARQ-RTT-TimerUL (per UL HARQ process): the minimum duration before a UL HARQ retransmission grant is expected by the MAC entity;

-	drx-RetransmissionTimerSL (per sidelink process): the maximum duration until a grant for SL retransmission is received;

-	drx-HARQ-RTT-TimerSL (per sidelink process): the minimum duration before an SL retransmission grant is expected by the MAC entity;

-	drx-LastTransmissionUL (optional): the configuration to start drx-HARQ-RTT-TimerUL after the last transmission within a bundle;

-	ps-Wakeup (optional): the configuration to start associated drx-onDurationTimer in case DCP is monitored but not detected;

-	ps-TransmitOtherPeriodicCSI (optional): the configuration to report periodic CSI that is not L1-RSRP on PUCCH during the time duration indicated by drx-onDurationTimer in case DCP is configured but associated drx-onDurationTimer is not started;

-	ps-TransmitPeriodicL1-RSRP (optional): the configuration to transmit periodic CSI that is L1-RSRP on PUCCH during the time duration indicated by drx-onDurationTimer in case DCP is configured but associated drx-onDurationTimer is not started;

-	downlinkHARQ-FeedbackDisabled (optional): the configuration to disable HARQ feedback per DL HARQ process;

-	uplinkHARQ-Mode (optional): the configuration to set HARQmodeA or HARQmodeB per UL HARQ process;

-	disableCG-RetransmissionMonitoring (optional): the configuration to disable starting drx-HARQ-RTT-TimerUL for UL transmission over a configured uplink grant;

-	drx-TimeReferenceSFN (optional): the configuration to indicate how UE initializes DRX_SFN_COUNTER.

The following UE variable is used for the DRX operation if drx-NonIntegerLongCycleStartOffset is configured:

-	DRX_SFN_COUNTER: the counter that increments when SFN changes to 0. The maximum value of this counter is at least 65535.

Serving Cells of a MAC entity may be configured by RRC in two DRX groups with separate DRX parameters. When RRC does not configure a secondary DRX group, there is only one DRX group and all Serving Cells belong to that one DRX group. When two DRX groups are configured, each Serving Cell is uniquely assigned to either of the two groups. The DRX parameters that are separately configured for each DRX group are: drx-onDurationTimer, drx-InactivityTimer. The DRX parameters that are common to the DRX groups are: drx-SlotOffset, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-LongCycleStartOffset, drx-NonIntegerLongCycleStartOffset, drx-ShortCycle (optional), drx-NonIntegerShortCycle (optional), drx-ShortCycleTimer (optional), drx-HARQ-RTT-TimerDL, and drx-HARQ-RTT-TimerUL.

When DRX is configured, the Active Time for Serving Cells in a DRX group includes the time while:

-	drx-onDurationTimer or drx-InactivityTimer configured for the DRX group is running; or

-	drx-RetransmissionTimerDL, drx-RetransmissionTimerUL or drx-RetransmissionTimerSL is running on any Serving Cell in the DRX group; or

-	ra-ContentionResolutionTimer (as described in clause 5.1.5) or msgB-ResponseWindow (as described in clause 5.1.4a) is running; or

-	a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4 or 5.22.1.5). If this Serving Cell is part of a non-terrestrial network, the Active Time is started after the Scheduling Request transmission that is performed when the SR_COUNTER is 0 for all the SR configurations with pending SR(s) plus the UE-gNB RTT; or

-	a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clauses 5.1.4 and 5.1.4a); or

-	there is an ongoing RACH-less LTM cell switch; or

-	there is an ongoing RACH-less handover in a terrestrial network.

The following MAC timers are used for DRX operation in a non-terrestrial network:

-	HARQ-RTT-TimerDL-NTN (per DL HARQ process configured with HARQ feedback enabled): the minimum duration before a DL assignment for HARQ retransmission is expected by the MAC entity;

-	HARQ-RTT-TimerUL-NTN (per UL HARQ process configured with HARQModeA): the minimum duration before a UL HARQ retransmission grant is expected by the MAC entity.

When DRX is not configured and multicast DRX is configured for a G-RNTI or G-CS-RNTI, the MAC entity shall:

1>	monitor the PDCCH as specified in TS 38.213 [6];

1>	if a MAC PDU is received in a configured downlink assignment for unicast; or

1>	if the PDCCH indicates a DL unicast transmission:

2>	stop the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process.

When DRX is configured, the MAC entity shall:

1>	if a MAC PDU is received in a configured downlink assignment for unicast:

2>	if this Serving Cell is configured with downlinkHARQ-FeedbackDisabled:

…

2>	else:

3>	start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

NOTE 1a:	Void.

NOTE 1b:	Void.

2>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process;

2>	stop the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process.

1>	if a MAC PDU is transmitted in a configured uplink grant and LBT failure indication is not received from lower layers:

2>	if this Serving Cell is configured with uplinkHARQ-Mode:

…

2>	else:

3>	if disableCG-RetransmissionMonitoring is not configured for the configured uplink grant:

4>	if drx-LastTransmissionUL is configured:

5>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the last transmission (within a bundle) of the corresponding PUSCH transmission.

4>	else:

5>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first transmission (within a bundle) of the corresponding PUSCH transmission.

2>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process at the first transmission (within a bundle) of the corresponding PUSCH transmission.

…

1>	if a drx-HARQ-RTT-TimerDL expires:

2>	if the data of the corresponding HARQ process was not successfully decoded:

3>	start the drx-RetransmissionTimerDL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerDL.

…

1>	if a drx-HARQ-RTT-TimerUL expires:

2>	start the drx-RetransmissionTimerUL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerUL.

…

1>	if a DRX Command MAC CE indicated by PDCCH addressed to C-RNTI or CS-RNTI, or by a configured downlink assignment for unicast transmission or a Long DRX Command MAC CE is received:

2>	stop drx-onDurationTimer for each DRX group;

2>	stop drx-InactivityTimer for each DRX group.

1>	if drx-InactivityTimer for a DRX group expires:

2>	if the Short DRX cycle is configured:

3>	start or restart drx-ShortCycleTimer for this DRX group in the first symbol after the expiry of drx-InactivityTimer;

3>	use the Short DRX cycle for this DRX group.

2>	else:

3>	use the Long DRX cycle for this DRX group.

1>	if a DRX Command MAC CE indicated by PDCCH addressed to C-RNTI or CS-RNTI, or by a configured downlink assignment for unicast transmission is received:

2>	if the Short DRX cycle is configured:

3>	start or restart drx-ShortCycleTimer for each DRX group in the first symbol after the end of DRX Command MAC CE reception;

3>	use the Short DRX cycle for each DRX group.

2>	else:

3>	use the Long DRX cycle for each DRX group.

1>	if drx-ShortCycleTimer for a DRX group expires:

2>	use the Long DRX cycle for this DRX group.

1>	if a Long DRX Command MAC CE is received:

2>	stop drx-ShortCycleTimer for each DRX group;

2>	use the Long DRX cycle for each DRX group.

1>	if the drx-NonIntegerLongCycleStartOffset is configured:

2>	increment DRX_SFN_COUNTER by 1 in the first symbol of a slot in which SFN changes to 0;

2>	if DRX is (re-)configured by RRC:

3>	if drx-TimeReferenceSFN is included in the RRC (re-)configuration which is received during the first half of a hyper frame (i.e., SFN is between 0 and 511):

4>	set DRX_SFN_COUNTER to 1.

3>	else:

4>	set DRX_SFN_COUNTER to 0.

1>	if the Short DRX cycle is used for a DRX group and the drx-NonIntegerShortCycle is not configured, and [(SFN × 10) + subframe number] modulo (drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle); or

1>	if the Short DRX cycle is used for a DRX group and the drx-NonIntegerShortCycle is configured, and floor([(DRX_SFN_COUNTER × 10240) + (SFN × 10) + subframe number - drx-StartOffset] modulo (drx-NonIntegerShortCycle)) = 0:

2>	start drx-onDurationTimer for this DRX group after drx-SlotOffset from the beginning of the subframe.

1>	if the Long DRX cycle is used for a DRX group and the drx-NonIntegerLongCycleStartOffset is not configured, and [(SFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset; or

1>	if the Long DRX cycle is used for a DRX group and the drx-NonIntegerLongCycleStartOffset is configured, and floor([(DRX_SFN_COUNTER × 10240) + (SFN × 10) + subframe number] modulo (drx-NonIntegerLongCycle)) = drx-StartOffset:

2>	if DCP monitoring is configured for the active DL BWP as specified in TS 38.213 [6], clause 10.3:

3>	if DCP indication associated with the current DRX cycle received from lower layer indicated to start drx-onDurationTimer, as specified in TS 38.213 [6]; or

3>	if all DCP occasion(s) in time domain, as specified in TS 38.213 [6], associated with the current DRX cycle occurred in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to start of the last DCP occasion, or during a measurement gap, or when the MAC entity monitors for a PDCCH transmission on the search space indicated by recoverySearchSpaceId of the SpCell identified by the C-RNTI while the ra-ResponseWindow is running (as specified in clause 5.1.4); or

3>	if ps-Wakeup is configured with value true and DCP indication associated with the current DRX cycle has not been received from lower layers:

4>	start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe.

2>	else:

3>	start drx-onDurationTimer for this DRX group after drx-SlotOffset from the beginning of the subframe.

NOTE 2:	In case of unaligned SFN across carriers in a cell group, the SFN of the SpCell is used to calculate the DRX duration.

1>	if a DRX group is in Active Time:

2>	monitor the PDCCH on the Serving Cells in this DRX group as specified in TS 38.213 [6];

2>	if the PDCCH indicates a DL transmission; or

2>	if the PDCCH indicates a one-shot HARQ feedback as specified in clause 9.1.4 of TS 38.213 [6]; or

2>	if the PDCCH indicates a retransmission of HARQ feedback as specified in clause 9.1.5 of TS 38.213 [6]:

3>	if this Serving Cell is configured with downlinkHARQ-FeedbackDisabled:

4>	if the corresponding HARQ process is configured with HARQ feedback enabled:

5>	set HARQ-RTT-TimerDL-NTN for the corresponding HARQ process equal to drx-HARQ-RTT-TimerDL plus the latest available UE-gNB RTT value;

5>	start the HARQ-RTT-TimerDL-NTN for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

3>	else:

4>	start or restart the drx-HARQ-RTT-TimerDL for the corresponding HARQ process(es) whose HARQ feedback is reported in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

NOTE 3:	When HARQ feedback is postponed by PDSCH-to-HARQ_feedback timing indicating an inapplicable k1 value, as specified in TS 38.213 [6], the corresponding transmission opportunity to send the DL HARQ feedback is indicated in a later PDCCH requesting the HARQ-ACK feedback.

3>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process(es) whose HARQ feedback is reported;

3>	stop the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process;

3>	if the PDSCH-to-HARQ_feedback timing indicate an inapplicable k1 value as specified in TS 38.213 [6]:

4>	start the drx-RetransmissionTimerDL in the first symbol after the (end of the last) PDSCH transmission (within a bundle) for the corresponding HARQ process.

2>	if the PDCCH indicates a UL transmission:

3>	if this Serving Cell is configured with uplinkHARQ-Mode:

4>	if the corresponding HARQ process is configured as HARQModeA:

5>	set HARQ-RTT-TimerUL-NTN for the corresponding HARQ process equal to drx-HARQ-RTT-TimerUL plus the latest available UE-gNB RTT value;

5>	if drx-LastTransmissionUL is configured:

6>	start the HARQ-RTT-TimerUL-NTN for the corresponding HARQ process in the first symbol after the end of the last transmission (within a bundle) of the corresponding PUSCH transmission.

5>	else:

6>	start the HARQ-RTT-TimerUL-NTN for the corresponding HARQ process in the first symbol after the end of the first transmission (within a bundle) of the corresponding PUSCH transmission.

3>	else:

4>	if drx-LastTransmissionUL is configured:

5>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the last transmission (within a bundle) of the corresponding PUSCH transmission.

4>	else:

5>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first transmission (within a bundle) of the corresponding PUSCH transmission.

3>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process.

2>	if the PDCCH indicates an SL transmission:

3>	if the PUCCH resource is configured:

4>	start the drx-HARQ-RTT-TimerSL for the corresponding HARQ process in the first symbol after the end of the corresponding PUCCH transmission carrying the SL HARQ feedback; or

4>	start the drx-HARQ-RTT-TimerSL for the corresponding HARQ process in the first symbol after the end of the corresponding PUCCH resource for the SL HARQ feedback when the PUCCH is not transmitted;

4>	stop the drx-RetransmissionTimerSL for the corresponding HARQ process.

3>	else:

4>	start the drx-HARQ-RTT-TimerSL for the corresponding HARQ process at the first symbol after end of PDCCH occasion;

4>	stop the drx-RetransmissionTimerSL for the corresponding HARQ process.

2>	if the PDCCH indicates a new transmission (DL, UL or SL) on a Serving Cell in this DRX group:

3>	start or restart drx-InactivityTimer for this DRX group in the first symbol after the end of the PDCCH reception.

NOTE 3a:	A PDCCH indicating activation of SPS, configured grant type 2, or configured sidelink grant of configured grant Type 2 is considered to indicate a new transmission.

NOTE 3b:	If the PDCCH reception includes two PDCCH candidates from corresponding search spaces, as described in clause 10.1 in 38.213, start or restart drx-InactivityTimer for this DRX group in the first symbol after the end of the PDCCH candidate that ends later in time.

2>	if a HARQ process receives downlink feedback information and acknowledgement is indicated:

3>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process.

1>	if DCP monitoring is configured for the active DL BWP as specified in TS 38.213 [6], clause 10.3; and

1>	if the current symbol n occurs within drx-onDurationTimer duration; and

1>	if drx-onDurationTimer associated with the current DRX cycle is not started as specified in this clause:

2>	if the MAC entity would not be in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause; and

2>	if allowCSI-SRS-Tx-MulticastDRX-Active is not configured, or if cfr-ConfigMulticast is not configured for any of the active BWP(s) of the Serving Cell(s), or if all multicast DRXes would not be in Active Time considering multicast assignments/DRX Command MAC CE for MBS multicast received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in Clause 5.7b and all multicast sessions are configured with multicast DRX:

3>	not transmit periodic SRS and semi-persistent SRS defined in TS 38.214 [7];

3>	not report semi-persistent CSI configured on PUSCH;

3>	not report semi-persistent CSI on PUCCH;

3>	if ps-TransmitPeriodicL1-RSRP is not configured with value true:

4>	not report periodic CSI that is L1-RSRP on PUCCH.

3>	if ps-TransmitOtherPeriodicCSI is not configured with value true:

4>	not report periodic CSI that is not L1-RSRP on PUCCH.

1>	else:

2>	in current symbol n, if a DRX group would not be in Active Time considering grants/assignments scheduled on Serving Cell(s) in this DRX group and DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause; and

2>	if allowCSI-SRS-Tx-MulticastDRX-Active is not configured, or if cfr-ConfigMulticast is not configured for any of the active BWP(s) of the Serving Cell(s), or, in current symbol n, if all multicast DRXes corresponding to the DRX group would not be in Active Time considering multicast assignments/DRX Command MAC CE for MBS multicast received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in Clause 5.7b and all multicast sessions corresponding to the DRX group are configured with multicast DRX:

3>	not transmit periodic SRS and semi-persistent SRS defined in TS 38.214 [7] in this DRX group;

3>	not report CSI on PUCCH and semi-persistent CSI configured on PUSCH in this DRX group.

2>	if CSI masking (csi-Mask) is setup by upper layers:

3>	in current symbol n, if drx-onDurationTimer of a DRX group would not be running considering grants/assignments scheduled on Serving Cell(s) in this DRX group and DRX Command MAC CE/Long DRX Command MAC CE received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause; and

3>	if allowCSI-SRS-Tx-MulticastDRX-Active is not configured, or if cfr-ConfigMulticast is not configured for any of the active BWP(s) of the Serving Cell(s), or, in current symbol n, if drx-onDurationTimerPTM(s) of all multicast DRXes corresponding to the DRX group would not be running considering DRX Command MAC CE for MBS multicast received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in Clause 5.7b and all multicast sessions corresponding to the DRX group are configured with multicast DRX:

4>	not report CSI on PUCCH in this DRX group.

NOTE 4:	If a UE multiplexes a CSI configured on PUCCH with other overlapping UCI(s) according to the procedure specified in TS 38.213 [6] clause 9.2.5 and this CSI multiplexed with other UCI(s) would be reported on a PUCCH resource either outside DRX Active Time of the DRX group in which this PUCCH is configured or outside the on-duration period of the DRX group in which this PUCCH is configured if CSI masking is setup by upper layers, it is up to UE implementation whether to report this CSI multiplexed with other UCI(s).

The MAC entity shall ensure no rounding error is generated when performing the modulus operation with drx-NonIntegerShortCycle or drx-NonIntegerLongCycle as the divisor.

Regardless of whether the MAC entity is monitoring PDCCH or not on the Serving Cells in a DRX group, the MAC entity transmits HARQ feedback, aperiodic CSI on PUSCH, and aperiodic SRS defined in TS 38.214 [7] on the Serving Cells in the DRX group when such is expected.

The MAC entity needs not to monitor the PDCCH if it is not a complete PDCCH occasion (e.g. the Active Time starts or ends in the middle of a PDCCH occasion).

7.1.1.5.9.3	Test description

7.1.1.5.9.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that set to return no data in uplink.

7.1.1.5.9.3.2	Test procedure sequence

Table 7.1.1.5.9.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | SS transmits RRCReconfiguration to configure specific DRX parameters. | <-- | RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits NR RRCReconfigurationComplete message. | --> | RRC: RRCReconfigurationComplete | - | - |
| 3 | In the first PDCCH occasion, after the drx-SlotOffset when the drx-onDurationTimer is running, the SS indicates the transmission of a DL MAC PDU on the PDCCH. | <-- | MAC PDU | - | - |
| 4 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 3? | --> | HARQ ACK | - | - |
| 5 | In the first PDCCH occasion after the drx-SlotOffset when the drx-onDurationTimer of drx-NonIntegerLongCycle is running at the first half of the hyperframe, the SS transmits RRCReconfiguration with drx-TimeReferenceSFN parameter. | <-- | RRC: RRCReconfiguration | - | - |
| 6 | The UE transmits RRCReconfigurationComplete message. | --> | RRC: RRCReconfigurationComplete | - | - |
| 7 | In the first PDCCH occasion after the drx-SlotOffset when the drx-onDurationTimer of drx-NonIntegerLongCycle is running, the SS indicates the transmission of a DL MAC PDU on the PDCCH. (Note 1) | <-- | MAC PDU | - | - |
| 8 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 8? | --> | HARQ ACK | 1 | P |
| 9 | In the first PDCCH occasion after the drx-SlotOffset when the drx-onDurationTimer of drx-NonIntegerLongCycle is running at the first half of the hyperframe, the SS transmits RRCReconfiguration without drx-TimeReferenceSFN parameter. | <-- | RRC: RRCReconfiguration | - | - |
| 10 | The UE transmits RRCReconfigurationComplete message. | --> | RRC: RRCReconfigurationComplete | - | - |
| 11 | In the first PDCCH occasion after the drx-SlotOffset when the drx-onDurationTimer of drx-NonIntegerLongCycle is running, the SS indicates the transmission of a DL MAC PDU on the PDCCH. (Note 2) | <-- | MAC PDU | - | - |
| 12 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 12? | --> | HARQ ACK | 2 | P |
| Note 1: This should happen when floor([10240 + (SFN × 10) + subframe number] modulo (drx-NonIntegerLongCycle)) = drx-StartOffset after the RRCReconfigurationComplete. Note 2: This should happen when floor([(SFN × 10) + subframe number] modulo (drx-NonIntegerLongCycle)) = drx-StartOffset after the RRCReconfigurationComplete. |  |  |  |  |  |

7.1.1.5.9.3.3	Specific message contents

Table 7.1.1.5.9.3.3-1: RRCReconfiguration (steps 1,5 and 9, Table 7.1.1.5.9.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-13

Table 7.1.1.5.9.3.3-2: CellGroupConfig (Table 7.1.1.5.9.3.3-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.3-19

Table 7.1.1.5.9.3.3-3: MAC-CellGroupConfig (Step 1, Table 7.1.1.5.9.3.3-2)

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MAC-CellGroupConfig ::= SEQUENCE { |  |  |  |
| drx-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| drx-onDurationTimer | ms2 |  |  |
| drx-InactivityTimer | ms1 |  |  |
| drx-HARQ-RTT-TimerDL | 56 | Number of slots=4 due to number of symbol per slot=14 | µ = 0,1,2,3,4 (2 with normal CP) |
|  | 48 | Number of slots=4 due to number of symbol per slot=12 | µ = 2 with extended CP |
| drx-HARQ-RTT-TimerUL | 56 | Number of slots=4 due to number of symbol per slot=14 | µ = 0,1,2,3,4 (2 with normal CP) |
|  | 48 | Number of slots=4 due to number of symbol per slot=12 | µ = 2 with extended CP |
| drx-RetransmissionTimerDL | sl8 |  |  |
| drx-RetransmissionTimerUL | sl8 |  |  |
| drx-LongCycleStartOffset CHOICE { |  |  |  |
| ms640 | 7 |  |  |
| } |  |  |  |
| shortDRX | Not present |  |  |
| drx-SlotOffset | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| drx-ConfigExt2-v1800 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| drx-NonIntegerLongCycleStartOffset-r18 CHOICE { |  |  |  |
| ms25over3 | 0 |  |  |
| } |  |  |  |
| drx-TimeReferenceSFN-r18 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.5.9.3.3-4: MAC-CellGroupConfig (Step 5, Table 7.1.1.5.9.3.3-2)

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MAC-CellGroupConfig ::= SEQUENCE { |  |  |  |
| drx-ConfigExt2-v1800 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| drx-NonIntegerLongCycleStartOffset-r18 CHOICE { |  |  |  |
| ms25over3 | 0 |  |  |
| } |  |  |  |
| drx-TimeReferenceSFN-r18 | sfn512 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.5.9.3.3-5: MAC-CellGroupConfig (Step 9, Table 7.1.1.5.9.3.3-2)

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MAC-CellGroupConfig ::= SEQUENCE { |  |  |  |
| drx-ConfigExt2-v1800 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| drx-NonIntegerLongCycleStartOffset-r18 CHOICE { |  |  |  |
| ms25over3 | 0 |  |  |
| } |  |  |  |
| drx-TimeReferenceSFN-r18 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 7.1.1.5.10	NR NTN / DRX operation / Short cycle not configured / Correct HARQ process handling / HARQ feedback disabled / HARQ modeB

7.1.1.5.10.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with one of the HARQ processes configured with HARQ feedback disabled }

ensure that {

when { UE successfully decodes data on two HARQ processes }

then { UE does not transmit HARQ feedback in the uplink for HARQ process configured with feedback disabled while feedback is transmitted for the other HARQ process }

}

(2)

with { UE in RRC_CONNECTED state with one of the HARQ processes configured as HARQModeB and receives data on two HARQ processes }

ensure that {

when { UE receives an UL Grant with NDI not toggled on two HARQ processes }

then { UE does not retransmit the data in uplink for the HARQ process configured with HARQModeB and re-transmits data for the other HARQ process }

}

7.1.1.5.10.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.321, clause 5.7. Unless otherwise stated these are Rel-17 requirements.

[TS 38.321, clause 5.7]

The MAC entity may be configured by RRC with a DRX functionality that controls the UE's PDCCH monitoring activity for the MAC entity's C-RNTI, CI-RNTI, CS-RNTI, INT-RNTI, SFI-RNTI, SP-CSI-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, TPC-SRS-RNTI, AI-RNTI, SL-RNTI, SL-CS-RNTI and SL Semi-Persistent Scheduling V-RNTI. When using DRX operation, the MAC entity shall also monitor PDCCH according to requirements found in other clauses of this specification. When in RRC_CONNECTED, if DRX is configured, for all the activated Serving Cells, the MAC entity may monitor the PDCCH discontinuously using the DRX operation specified in this clause; otherwise the MAC entity shall monitor the PDCCH as specified in TS 38.213 [6].

NOTE 1:	Void

RRC controls DRX operation by configuring the following parameters:

-	drx-onDurationTimer: the duration at the beginning of a DRX cycle;

-	drx-SlotOffset: the delay before starting the drx-onDurationTimer;

-	drx-InactivityTimer: the duration after the PDCCH occasion in which a PDCCH indicates a new UL, DL or SL transmission for the MAC entity;

-	drx-RetransmissionTimerDL (per DL HARQ process except for the broadcast process): the maximum duration until a DL retransmission is received;

-	drx-RetransmissionTimerUL (per UL HARQ process): the maximum duration until a grant for UL retransmission is received;

-	drx-LongCycleStartOffset: the Long DRX cycle and drx-StartOffset which defines the subframe where the Long and Short DRX cycle starts;

-	drx-ShortCycle (optional): the Short DRX cycle;

-	drx-ShortCycleTimer (optional): the duration the UE shall follow the Short DRX cycle;

-	drx-HARQ-RTT-TimerDL (per DL HARQ process except for the broadcast process): the minimum duration before a DL assignment for HARQ retransmission is expected by the MAC entity;

-	drx-HARQ-RTT-TimerUL (per UL HARQ process): the minimum duration before a UL HARQ retransmission grant is expected by the MAC entity;

…

-	downlinkHARQ-FeedbackDisabled (optional): the configuration to disable HARQ feedback per DL HARQ process;

-	uplinkHARQ-Mode (optional): the configuration to set HARQmodeA or HARQmodeB per UL HARQ process.

Serving Cells of a MAC entity may be configured by RRC in two DRX groups with separate DRX parameters. When RRC does not configure a secondary DRX group, there is only one DRX group and all Serving Cells belong to that one DRX group. When two DRX groups are configured, each Serving Cell is uniquely assigned to either of the two groups. The DRX parameters that are separately configured for each DRX group are: drx-onDurationTimer, drx-InactivityTimer. The DRX parameters that are common to the DRX groups are: drx-SlotOffset, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-LongCycleStartOffset, drx-ShortCycle (optional), drx-ShortCycleTimer (optional), drx-HARQ-RTT-TimerDL, and drx-HARQ-RTT-TimerUL.

When DRX is configured, the Active Time for Serving Cells in a DRX group includes the time while:

-	drx-onDurationTimer or drx-InactivityTimer configured for the DRX group is running; or

-	drx-RetransmissionTimerDL, drx-RetransmissionTimerUL or drx-RetransmissionTimerSL is running on any Serving Cell in the DRX group; or

-	ra-ContentionResolutionTimer (as described in clause 5.1.5) or msgB-ResponseWindow (as described in clause 5.1.4a) is running; or

-	a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4 or 5.22.1.5). If this Serving Cell is part of a non-terrestrial network, the Active Time is started after the Scheduling Request transmission that is performed when the SR_COUNTER is 0 for all the SR configurations with pending SR(s) plus the UE-gNB RTT; or

-	a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clauses 5.1.4 and 5.1.4a).

The following MAC timers are used for DRX operation in a non-terrestrial network:

-	HARQ-RTT-TimerDL-NTN (per DL HARQ process configured with HARQ feedback enabled): the minimum duration before a DL assignment for HARQ retransmission is expected by the MAC entity;

-	HARQ-RTT-TimerUL-NTN (per UL HARQ process configured with HARQModeA): the minimum duration before a UL HARQ retransmission grant is expected by the MAC entity.

…

When DRX is configured, the MAC entity shall:

1>	if a MAC PDU is received in a configured downlink assignment for unicast:

2>	if this Serving Cell is configured with downlinkHARQ-FeedbackDisabled:

3>	if the corresponding HARQ process is configured with HARQ feedback enabled:

4>	set HARQ-RTT-TimerDL-NTN for the corresponding HARQ process equal to drx-HARQ-RTT-TimerDL plus the latest available UE-gNB RTT value;

4>	start the HARQ-RTT-TimerDL-NTN for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

2>	else:

3>	start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

NOTE 1a:	Void.

NOTE 1b:	Void.

2>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process;

2>	stop the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process.

1>	if a MAC PDU is transmitted in a configured uplink grant and LBT failure indication is not received from lower layers:

2>	if this Serving Cell is configured with uplinkHARQ-Mode:

3>	if the corresponding HARQ process is configured as HARQModeA:

4>	set HARQ-RTT-TimerUL-NTN for the corresponding HARQ process equal to drx-HARQ-RTT-TimerUL plus the latest available UE-gNB RTT value;

4>	if drx-LastTransmissionUL is configured:

5>	start the HARQ-RTT-TimerUL-NTN for the corresponding HARQ process in the first symbol after the end of the last transmission (within a bundle) of the corresponding PUSCH transmission.

4>	else:

5>	start the HARQ-RTT-TimerUL-NTN for the corresponding HARQ process in the first symbol after the end of the first transmission (within a bundle) of the corresponding PUSCH transmission.

2>	else:

3>	if drx-LastTransmissionUL is configured:

4>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the last transmission (within a bundle) of the corresponding PUSCH transmission.

3>	else:

4>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first transmission (within a bundle) of the corresponding PUSCH transmission.

2>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process at the first transmission (within a bundle) of the corresponding PUSCH transmission.

…

1>	if a drx-HARQ-RTT-TimerDL expires:

2>	if the data of the corresponding HARQ process was not successfully decoded:

3>	start the drx-RetransmissionTimerDL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerDL.

1>	if a HARQ-RTT-TimerDL-NTN expires:

2>	if the data of the corresponding HARQ process was not successfully decoded:

3>	start the drx-RetransmissionTimerDL for the corresponding HARQ process in the first symbol after the expiry of HARQ-RTT-TimerDL-NTN.

1>	if a drx-HARQ-RTT-TimerUL expires:

2>	start the drx-RetransmissionTimerUL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerUL.

1>	if a HARQ-RTT-TimerUL-NTN expires:

2>	start the drx-RetransmissionTimerUL for the corresponding HARQ process in the first symbol after the expiry of HARQ-RTT-TimerUL-NTN.

…

1>	if a DRX Command MAC CE indicated by PDCCH addressed to C-RNTI or CS-RNTI, or by a configured downlink assignment for unicast transmission or a Long DRX Command MAC CE is received:

2>	stop drx-onDurationTimer for each DRX group;

2>	stop drx-InactivityTimer for each DRX group.

1>	if drx-InactivityTimer for a DRX group expires:

2>	if the Short DRX cycle is configured:

3>	start or restart drx-ShortCycleTimer for this DRX group in the first symbol after the expiry of drx-InactivityTimer;

3>	use the Short DRX cycle for this DRX group.

2>	else:

3>	use the Long DRX cycle for this DRX group.

1>	if a DRX Command MAC CE indicated by PDCCH addressed to C-RNTI or CS-RNTI, or by a configured downlink assignment for unicast transmission is received:

2>	if the Short DRX cycle is configured:

3>	start or restart drx-ShortCycleTimer for each DRX group in the first symbol after the end of DRX Command MAC CE reception;

3>	use the Short DRX cycle for each DRX group.

2>	else:

3>	use the Long DRX cycle for each DRX group.

1>	if drx-ShortCycleTimer for a DRX group expires:

2>	use the Long DRX cycle for this DRX group.

1>	if a Long DRX Command MAC CE is received:

2>	stop drx-ShortCycleTimer for each DRX group;

2>	use the Long DRX cycle for each DRX group.

1>	if the Short DRX cycle is used for a DRX group, and [(SFN × 10) + subframe number] modulo (drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle):

2>	start drx-onDurationTimer for this DRX group after drx-SlotOffset from the beginning of the subframe.

1>	if the Long DRX cycle is used for a DRX group, and [(SFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset:

2>	if DCP monitoring is configured for the active DL BWP as specified in TS 38.213 [6], clause 10.3:

3>	if DCP indication associated with the current DRX cycle received from lower layer indicated to start drx-onDurationTimer, as specified in TS 38.213 [6]; or

3>	if all DCP occasion(s) in time domain, as specified in TS 38.213 [6], associated with the current DRX cycle occurred in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to start of the last DCP occasion, or during a measurement gap, or when the MAC entity monitors for a PDCCH transmission on the search space indicated by recoverySearchSpaceId of the SpCell identified by the C-RNTI while the ra-ResponseWindow is running (as specified in clause 5.1.4); or

3>	if ps-Wakeup is configured with value true and DCP indication associated with the current DRX cycle has not been received from lower layers:

4>	start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe.

2>	else:

3>	start drx-onDurationTimer for this DRX group after drx-SlotOffset from the beginning of the subframe.

NOTE 2:	In case of unaligned SFN across carriers in a cell group, the SFN of the SpCell is used to calculate the DRX duration.

1>	if a DRX group is in Active Time:

2>	monitor the PDCCH on the Serving Cells in this DRX group as specified in TS 38.213 [6];

2>	if the PDCCH indicates a DL transmission; or

2>	if the PDCCH indicates a one-shot HARQ feedback as specified in clause 9.1.4 of TS 38.213 [6]; or

2>	if the PDCCH indicates a retransmission of HARQ feedback as specified in clause 9.1.5 of TS 38.213 [6]:

3>	if this Serving Cell is configured with downlinkHARQ-FeedbackDisabled:

4>	if the corresponding HARQ process is configured with HARQ feedback enabled:

5>	set HARQ-RTT-TimerDL-NTN for the corresponding HARQ process equal to drx-HARQ-RTT-TimerDL plus the latest available UE-gNB RTT value;

5>	start the HARQ-RTT-TimerDL-NTN for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

3>	else:

4>	start or restart the drx-HARQ-RTT-TimerDL for the corresponding HARQ process(es) whose HARQ feedback is reported in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback.

NOTE 3:	When HARQ feedback is postponed by PDSCH-to-HARQ_feedback timing indicating an inapplicable k1 value, as specified in TS 38.213 [6], the corresponding transmission opportunity to send the DL HARQ feedback is indicated in a later PDCCH requesting the HARQ-ACK feedback.

3>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process(es) whose HARQ feedback is reported;

3>	stop the drx-RetransmissionTimerDL-PTM for the corresponding HARQ process;

3>	if the PDSCH-to-HARQ_feedback timing indicate an inapplicable k1 value as specified in TS 38.213 [6]:

4>	start the drx-RetransmissionTimerDL in the first symbol after the (end of the last) PDSCH transmission (within a bundle) for the corresponding HARQ process.

2>	if the PDCCH indicates a UL transmission:

3>	if this Serving Cell is configured with uplinkHARQ-Mode:

4>	if the corresponding HARQ process is configured as HARQModeA:

5>	set HARQ-RTT-TimerUL-NTN for the corresponding HARQ process equal to drx-HARQ-RTT-TimerUL plus the latest available UE-gNB RTT value;

5>	if drx-LastTransmissionUL is configured:

6>	start the HARQ-RTT-TimerUL-NTN for the corresponding HARQ process in the first symbol after the end of the last transmission (within a bundle) of the corresponding PUSCH transmission.

5>	else:

6>	start the HARQ-RTT-TimerUL-NTN for the corresponding HARQ process in the first symbol after the end of the first transmission (within a bundle) of the corresponding PUSCH transmission.

3>	else:

4>	if drx-LastTransmissionUL is configured:

5>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the last transmission (within a bundle) of the corresponding PUSCH transmission.

4>	else:

5>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first transmission (within a bundle) of the corresponding PUSCH transmission.

3>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process.

…

Regardless of whether the MAC entity is monitoring PDCCH or not on the Serving Cells in a DRX group, the MAC entity transmits HARQ feedback, aperiodic CSI on PUSCH, and aperiodic SRS defined in TS 38.214 [7] on the Serving Cells in the DRX group when such is expected.

The MAC entity needs not to monitor the PDCCH if it is not a complete PDCCH occasion (e.g. the Active Time starts or ends in the middle of a PDCCH occasion).

7.1.1.5.10.3	Test description

7.1.1.5.10.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that DRB is configured in RLC AM mode according to Table 7.1.1.5.10.3.1-1.

Table 7.1.1.5.10.3.1-1: RLC parameters

| t-PollRetransmit | ms4000 |
| --- | --- |

System Simulator:

-	NR Cell 1 as specified in TS 38.508-1 [4] Clause 4.4.2.

-	System information combination NR-29 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used.

UE:

-	The pre-configured UE location is defined in TS 38.508-1 [4] Clause 4.5C.

7.1.1.5.10.3.2	Test procedure sequence

Table 7.1.1.5.10.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits RRCReconfiguration to configure specific DRX parameters. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits RRCReconfigurationComplete. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | During the whole test sequence, the SS does not allocate UL grants unless when explicitly stated so in the procedure. | - | - | - | - |
| 4 | In the first PDCCH occasion when the drx-onDurationTimer is running, the SS indicates transmission of a DL MAC PDU including an RLC PDU with poll bit not set on the PDCCH on HARQ ID 0. | <-- | MAC PDU | - | - |
| 5 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 4? | --> | HARQ ACK | 1 | P |
| 6 | In the next PDCCH occasion when the drx-onDurationTimer is running, the SS indicates the transmission of a DL MAC PDU including an RLC PDU with poll bit not set on the PDCCH on HARQ ID 1. | <-- | MAC PDU | - | - |
| - | EXCEPTION: Step 7a1 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that takes place depending on UE support of disabled HARQ feedback for downlink transmission. | - | - | - | - |
| 7a1 | IF pc_harq_FeedbackDisabled_r17, Check: Does the UE transmit a HARQ ACK/NACK for the DL MAC PDU in Step 6? | --> | HARQ ACK/NACK | 1 | F |
| 8 | In the last PDCCH occasion of subsequent DRX cycle when the drx-onDurationTimer is still running, the SS indicates an UL grant on PDCCH on HARQ ID 0 allowing UE to return one MAC PDU. | <-- | UL grant on PDCCH | - | - |
| 9 | UE loops back data received in step 4. | --> | MAC PDU | - | - |
| 10 | In the last PDCCH occasion of subsequent DRX cycle when the drx-onDurationTimer is still running, the SS indicates an UL grant on PDCCH on HARQ ID 1 allowing UE to return one MAC PDU. | <-- | UL grant on PDCCH | - | - |
| 11 | UE loops back data received in step 6. | --> | MAC PDU | - | - |
| 12 | In the next DRX cycle after Step 11, in the first PDCCH occasion when the drx-onDurationTimer is still running, the SS indicates an UL grant to the UE on HARQ ID 0 with NDI not toggled allowing the UE to retransmit MAC PDU at step 9. | <-- | UL grant on PDCCH | - | - |
| 13 | Check: Does the UE retransmit the MAC PDU in step 9? | --> | MAC PDU | 2 | P |
| 13A | SS transmits a MAC PDU containing an RLC STATUS PDU acknowledging the reception of the PDU in step 13. | <-- | MAC PDU (RLC STATUS PDU) | - | - |
| 14 | In the next DRX cycle after Step 13, in the first PDCCH occasion when the drx-onDurationTimer is still running, the SS indicates an UL grant on HARQ ID 1 with NDI not toggled allowing the UE to retransmit MAC PDU at step 11. | <-- | UL grant on PDCCH | - | - |
| - | EXCEPTION: Steps 15a1 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that takes place depending on UE support of HARQ mode B. | - | - | - | - |
| 15a1 | IF pc_ uplink_Harq_ModeB_r17, Check: Does the UE retransmit same data as in step 11? | --> | MAC PDU | 2 | F |
| 16 | SS transmits a MAC PDU containing an RLC STATUS PDU acknowledging the reception of the PDU in step 11. | <-- | MAC PDU (RLC STATUS PDU) | - | - |

7.1.1.5.10.3.3	Specific message contents

Table 7.1.1.5.10.3.3-1: RRCReconfiguration (step 1, Table 7.1.1.5.10.3.2-1)

| Derivation Path: 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.5.10.3.3-2: CellGroupConfig (Table 7.1.1.5.10.3.3-1)

| Derivation Path: 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| cellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-BearerConfig { | 1 entry |  |  |
| RLC-Bearer-Config[1] SEQUENCE { |  |  |  |
| mac-LogicalChannelConfig SEQUENCE { |  |  |  |
| ul-SpecificParameters SEQUENCE { |  |  |  |
| allowedHARQ-mode-r17 | harqModeB |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| mac-CellGroupConfig SEQUENCE { |  |  |  |
| drx-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| drx-onDurationTimer CHOICE { |  |  |  |
| milliSeconds | ms20 |  |  |
| } |  |  |  |
| drx-InactivityTimer | ms10 |  |  |
| drx-HARQ-RTT-TimerDL | 56 |  |  |
| drx-HARQ-RTT-TimerUL | 56 |  |  |
| drx-RetransmissionTimerDL | sl8 |  |  |
| drx-RetransmissionTimerUL | sl8 |  |  |
| drx-LongCycleStartOffset CHOICE { |  |  |  |
| ms640 | 7 |  |  |
| } |  |  |  |
| shortDRX | Not present |  |  |
| drx-SlotOffset | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated | ServingCellConfig |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.5.10.3.3-3: ServingCellConfig (Table 7.1.1.5.10.3.3-2)

| Derivation Path: 38.508-1 [4], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| pusch-ServingCellConfig CHOICE { |  |  |  |
| setup | PUSCH-ServingCellConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| pdsch-ServingCellConfig CHOICE { |  |  |  |
| setup | PDSCH-ServingCellConfig |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.5.10.3.3-4: PUSCH-ServingCellConfig (Table 7.1.1.5.10.3.3-3)

| Derivation Path: 38.508-1 [4], Table 4.6.3-121 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ServingCellConfig ::= SEQUENCE { |  |  |  |
| nrofHARQ-ProcessesForPUSCH-r17 | n32 |  |  |
| uplinkHARQ-mode-r17 CHOICE { |  |  | pc_uplink_Harq_ModeB_r17 |
| setup | ‘1000 0000 0000 0000 0000 0000 0000 0000’B | All HARQ processes configured with HARQ modeB except HARQ ID 0 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.5.10.3.3-5: PDSCH-ServingCellConfig (Table 7.1.1.5.10.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-102 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ServingCellConfig ::= SEQUENCE { |  |  |  |
| downlinkHARQ-FeedbackDisabled-r17 CHOICE { |  |  | pc_harq_FeedbackDisabled_r17 |
| setup | ‘0111 1111 1111 1111 1111 1111 1111 1111’B | HARQ feedback disabled for all HARQ processes except HARQ ID 0 |  |
| } |  |  |  |
| nrofHARQ-ProcessesForPDSCH-v1700 | n32 |  |  |
| } |  |  |  |

##### 7.1.1.5.11	DRX operation / Short cycle not configured / RACH-less LTM cell switch

7.1.1.5.11.1	Test Purpose (TP)

(1)

with { UE in RRC_Connected state with Long DRX configured and performs a RACH-less LTM cell switch }

ensure that {

when { UE performs a MAC Reset and applies candidate cell DRX configuration }

then { UE starts the OnDurationTimer and monitors the PDCCH for OnDurationTimer PDCCH-Occasions }

}

7.1.1.5.11.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.321 clauses 5.7, 5.18.35 , TS 38.331 clauses 5.3.5.18.1, 5.3.5.18.3, TS38.300 clause 9.2.3.5.3. Unless otherwise stated these are Rel-18 requirements.

[TS 38.321, clause 5.7]

The MAC entity may be configured by RRC with a DRX functionality that controls the UE's PDCCH monitoring activity for the MAC entity's C-RNTI, CI-RNTI, CS-RNTI, INT-RNTI, SFI-RNTI, SP-CSI-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, TPC-SRS-RNTI, AI-RNTI, SL-RNTI, SL-CS-RNTI, SL-PRS-RNTI, SL-PRS-CS-RNTI, SL Semi-Persistent Scheduling V-RNTI and cellDTRX-RNTI. When using DRX operation, the MAC entity shall also monitor PDCCH according to requirements found in other clauses of this specification. When in RRC_CONNECTED, if DRX is configured, for all the activated Serving Cells, the MAC entity may monitor the PDCCH discontinuously using the DRX operation specified in this clause; otherwise the MAC entity shall monitor the PDCCH as specified in TS 38.213 [6].

NOTE 1:	Void

RRC controls DRX operation by configuring the following parameters:

-	drx-onDurationTimer: the duration at the beginning of a DRX cycle;

-	drx-SlotOffset: the delay before starting the drx-onDurationTimer;

-	drx-InactivityTimer: the duration after the PDCCH occasion in which a PDCCH indicates a new UL, DL or SL transmission for the MAC entity;

-	drx-RetransmissionTimerDL (per DL HARQ process except for the broadcast process): the maximum duration until a DL retransmission is received;

-	drx-RetransmissionTimerUL (per UL HARQ process): the maximum duration until a grant for UL retransmission is received;

-	drx-LongCycleStartOffset: the Long DRX cycle and drx-StartOffset which defines the subframe where the Long and Short DRX cycle starts;

-	drx-NonIntegerLongCycleStartOffset (optional): the Long DRX cycle and drx-StartOffset which defines the subframe where the Long and Short DRX cycle start, when the length of the Long DRX cycle and/or the short DRX cycle is not an integer;

-	drx-ShortCycle (optional): the Short DRX cycle;

-	drx-NonIntegerShortCycle (optional): the Short DRX cycle whose length is not an integer;

-	drx-ShortCycleTimer (optional): the duration the UE shall follow the Short DRX cycle;

-	drx-HARQ-RTT-TimerDL (per DL HARQ process except for the broadcast process): the minimum duration before a DL assignment for HARQ retransmission is expected by the MAC entity;

-	drx-HARQ-RTT-TimerUL (per UL HARQ process): the minimum duration before a UL HARQ retransmission grant is expected by the MAC entity;

-	drx-RetransmissionTimerSL (per sidelink process): the maximum duration until a grant for SL retransmission is received;

-	drx-HARQ-RTT-TimerSL (per sidelink process): the minimum duration before an SL retransmission grant is expected by the MAC entity;

-	drx-LastTransmissionUL (optional): the configuration to start drx-HARQ-RTT-TimerUL after the last transmission within a bundle;

-	ps-Wakeup (optional): the configuration to start associated drx-onDurationTimer in case DCP is monitored but not detected;

-	ps-TransmitOtherPeriodicCSI (optional): the configuration to report periodic CSI that is not L1-RSRP on PUCCH during the time duration indicated by drx-onDurationTimer in case DCP is configured but associated drx-onDurationTimer is not started;

-	ps-TransmitPeriodicL1-RSRP (optional): the configuration to transmit periodic CSI that is L1-RSRP on PUCCH during the time duration indicated by drx-onDurationTimer in case DCP is configured but associated drx-onDurationTimer is not started;

-	downlinkHARQ-FeedbackDisabled (optional): the configuration to disable HARQ feedback per DL HARQ process;

-	uplinkHARQ-Mode (optional): the configuration to set HARQmodeA or HARQmodeB per UL HARQ process;

-	disableCG-RetransmissionMonitoring (optional): the configuration to disable starting drx-HARQ-RTT-TimerUL for UL transmission over a configured uplink grant;

-	drx-TimeReferenceSFN (optional): the configuration to indicate how UE initializes of DRX_SFN_COUNTER.

The following UE variable is used for the DRX operation if drx-NonIntegerLongCycleStartOffset is configured:

-	DRX_SFN_COUNTER: the counter that increments when SFN changes to 0. The maximum value of this counter is at least 65535.

Serving Cells of a MAC entity may be configured by RRC in two DRX groups with separate DRX parameters. When RRC does not configure a secondary DRX group, there is only one DRX group and all Serving Cells belong to that one DRX group. When two DRX groups are configured, each Serving Cell is uniquely assigned to either of the two groups. The DRX parameters that are separately configured for each DRX group are: drx-onDurationTimer, drx-InactivityTimer. The DRX parameters that are common to the DRX groups are: drx-SlotOffset, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-LongCycleStartOffset, drx-NonIntegerLongCycleStartOffset, drx-ShortCycle (optional), drx-NonIntegerShortCycle (optional), drx-ShortCycleTimer (optional), drx-HARQ-RTT-TimerDL, and drx-HARQ-RTT-TimerUL.

When DRX is configured, the Active Time for Serving Cells in a DRX group includes the time while:

-	drx-onDurationTimer or drx-InactivityTimer configured for the DRX group is running; or

-	drx-RetransmissionTimerDL, drx-RetransmissionTimerUL or drx-RetransmissionTimerSL is running on any Serving Cell in the DRX group; or

-	ra-ContentionResolutionTimer (as described in clause 5.1.5) or msgB-ResponseWindow (as described in clause 5.1.4a) is running; or

-	a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4 or 5.22.1.5). If this Serving Cell is part of a non-terrestrial network, the Active Time is started after the Scheduling Request transmission that is performed when the SR_COUNTER is 0 for all the SR configurations with pending SR(s) plus the UE-gNB RTT; or

-	a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clauses 5.1.4 and 5.1.4a); or

-	there is an ongoing RACH-less LTM cell switch; or

-	there is an ongoing RACH-less handover in a terrestrial network.

…

[TS 38.331, clause 5.3.5.3]

The UE shall perform the following actions upon reception of the RRCReconfiguration, upon execution of the conditional reconfiguration (CHO, CPA, CPC, or subsequent CPAC), or upon execution of an LTM cell switch:

…

1>	if the RRCReconfiguration message includes the ltm-Config:

2>	if the ltm-Config is set to setup:

3>	perform the LTM configuration procedure as specified in 5.3.5.18.1;

2>	else:

3>	perform the LTM configuration release procedure as specified in clause 5.3.5.18.7;

…

2>	if this RRCReconfiguration message is applied due to an LTM cell switch execution procedure according to clause 5.3.5.18.6:

3>	include in the appliedLTM-CandidateId the LTM-CandidateId of the applied LTM candidate configuration;

…

[TS 38.331, clause 5.3.5.18.1]

The network configures the UE with one or more LTM candidate configurations within the LTM-Config IE.

...

The UE shall perform the following actions based on the received LTM-Config IE:

1>	if the received LTM-Config includes ltm-ServingCellNoResetID:

2>	if the current VarLTM-ServingCellNoResetID includes an ltm-ServingCellNoResetID:

3>	replace the ltm-ServingCellNoResetID value within VarLTM-ServingCellNoResetID with the received ltm-ServingCellNoResetID;

2>	else:

3>	store the received ltm-ServingCellNoResetID in VarLTM-ServingCellNoResetID;

1>	if the received LTM-Config includes ltm-ServingCellUE-MeasuredTA-ID:

2>	if the current VarLTM-ServingCellUE-MeasuredTA-ID includes an ltm-ServingCellUE-MeasuredTA-ID:

3>	replace the ltm-ServingCellUE-MeasuredTA-ID value within VarLTM-ServingCellUE-MeasuredTA-ID with the received ltm-ServingCellUE-MeasuredTA-ID;

2>	else:

3>	store the received ltm-ServingCellUE-MeasuredTA-ID in VarLTM-ServingCellUE-MeasuredTA-ID;

1>	if the received LTM-Config includes the ltm-CandidateToReleaseList:

2>	perform the LTM candidate configuration release as specified in 5.3.5.18.2;

1>	if the received LTM-Config includes the ltm-CandidateToAddModList:

2>	perform the LTM candidate configuration addition or modification as specified in 5.3.5.18.3;

1>	reconfigure the UE according to all other fields of the received LTM-Config IE.

[TS 38.321, clause 5.18.35]

The network may instruct the UE to perform LTM cell switch procedure by sending the LTM Cell Switch Command MAC CE described in clause 6.1.3.75.

The MAC entity shall:

1>	if the MAC entity receives an LTM Cell Switch Command MAC CE on a Serving Cell:

2>	indicate to upper layers that the LTM cell switch procedure is triggered and the Target Configuration ID included in the LTM Cell Switch Command MAC CE;

2>	if the MAC reset operation as specified in clause 5.12 is performed, as requested by upper layers:

3>	if Timing Advance Command value (hexa-decimal) is not set as FFF:

4>	process the received Timing Advance Command (see clause 5.2);

4>	consider the RACH-less LTM cell switch to be ongoing;

4>	if the MAC entity is associated with the SCG:

5>	indicate to upper layers to skip the Random Access procedure for this LTM cell switch.

3>	else if the UE is configured with UE-based Timing Advance measurement as specified in TS 38.331 [5] and the UE has successfully measured the Timing Advance for the SpCell of the indicated LTM target configuration:

4>	process the measured Timing Advance (see clause 5.2);

4>	consider the RACH-less LTM cell switch to be ongoing.

4>	if the MAC entity is associated with the SCG:

5>	indicate to upper layers to skip the Random Access procedure for this LTM cell switch.

3>	indicate to lower layers the information regarding the TCI state information included in the LTM Cell Switch Command MAC CE.

[TS 38.331, clause 5.3.5.18.3]

The UE shall:

1>	for each ltm-CandidateId value included in the ltm-CandidateToAddModList:

2>	if the current UE configuration contains an LTM-Candidate with the ltm-CandidateId value:

3>	reconfigure the corresponding LTM-Candidate in accordance with the received LTM-Candidate;

2>	else:

3>	add the received LTM-Candidate;

2>	if the LTM-Candidate with the received ltm-CandidateId value includes ltm-UE-MeasuredTA-ID:

3>	if the value of ltm-UE-MeasuredTA-ID is equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID:

4>	inform lower layers that the UE is configured with UE-based TA measurements for this LTM-Candidate;

3>	else:

4>	inform lower layers that the UE is not configured with UE-based TA measurements for this LTM-Candidate;

2>	else:

3>	inform lower layers that the UE is not configured with UE-based TA measurements for this LTM-Candidate.

…

1>	if the LTM cell switch is triggered by an indication from lower layers:

2>	apply the RRCReconfiguration message in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config identified by the LTM candidate configuration identity received from lower layers according to clause 5.3.5.3;

1>	else (LTM cell switch triggered upon cell selection performed while timer T311 was running):

2>	apply the RRCReconfiguration message in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config related to the LTM candidate configuration identity for the selected cell (i.e., in accordance with 5.3.7.3) according to clause 5.3.5.3;

1>	release the radio bearer(s) and the logical channel(s) that were part of the UE configuration before of this LTM cell switch procedure but not part of the LTM candidate configuration either indicated by lower layers or for the selected cell in accordance with 5.3.7.3, or the LTM reference configuration (in case the LTM candidate configuration does not include ltm-ConfigComplete).

NOTE 2:	When ltm-ConfigComplete is not included for an LTM candidate configuration, before an LTM cell switch is triggered a UE implementation may generate and store an RRCReconfiguration message by applying the received LTM candidate configuration on top of the LTM reference configuration, and the stored RRCReconfiguration message is applied when the LTM cell switch is triggered. It is up to the UE to ensure that the RRC reconfiguration applied at the time of LTM cell switch is in accordance with the latest LTM reference configuration and LTM candidate configuration.

[TS 38.300, clause 9.2.3.5.3]

After receiving an LTM cell switch command MAC CE, the UE performs MAC reset. Whether the UE performs RLC re-establishment and PDCP data recovery during cell switch is explicitly controlled by the network through RRC signalling.

7.1.1.5.11.3	Test description

7.1.1.5.11.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that System information combination NR-2 except UE is set to return no data in uplink, System information combination NR-2 is broadcasted in both cells and NR Cell 2 is configured as an active cell.

7.1.1.5.11.3.2	Test procedure sequence

Table 7.1.1.5.11.3.2-1 and 7.1.1.5.11.3.2-2 illustrates the downlink power levels to be applied for NR Cell 1 and NR Cell 2 at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while the configuration marked "T1" is applied at the point indicated in the Main behaviour description in Table 7.1.1.5.11.3.2-3.

Table 7.1.1.5.11.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -85 | -91 | Power levels are such that entry condition for event A3 is not satisfied for the neighbour NR cell |
| T1 | SS/PBCH SSS EPRE | dBm/ SCS | -85 | -79 | Power levels are such that entry condition for event A3 is satisfied for intra-frequency neighbour NR cell |

Table 7.1.1.5.11.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -91 | -100 | Power levels are such that entry condition for event A3 is not satisfied for the neighbour NR cell |
| T1 | SS/PBCH SSS EPRE | dBm/ SCS | -91 | -82 | Power levels are such that entry condition for event A3 is satisfied for intra-frequency neighbour NR cell |

Table 7.1.1.5.11.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message including MeasConfig to setup NR measurement and reporting for intra-frequency event A3 on NR Cell 1. | <-- | RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message. | --> | RRCReconfigurationComplete | - | - |
| 3 | SS re-adjusts the cell-specific reference signal level according to row "T1" in table 7.1.1.5.11.3.2-2 | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message to report event A3 with the measured RSRP value for NR Cell 2 in NR Cell 1. | --> | MeasurementReport | - | - |
| 5 | The SS transmits an RRCReconfiguration message with LTM-Candidate-r18 for NR Cell 2 on NR Cell 1. | <-- | RRCReconfiguration | - | - |
| 6 | The UE transmits RRCReconfigurationComplete message on NR Cell 1. | --> | RRCReconfigurationComplete | - | - |
| 7 | The SS transmits the candidate cell TCI activation MAC CE with Candidate Cell ID set to 0 for NR Cell 2 on NR Cell 1. | <-- | MAC PDU (Candidate Cell TCI Activation/Deactivation MAC CE) | - | - |
| 8 | The SS transmits an LTM Cell Switch Command MAC CE for switching to the candidate cell with Timing Advance Command value set to 0 on NR Cell1 to the UE | <-- | MAC PDU (LTM Cell Switch Command MAC CE) | - | - |
| 9 | The SS transmits a periodic UL Grant to the UE for uplink transmission on NR Cell 2 | <-- | UL Grant (C-RNTI) | - | - |
| 10 | The SS transmits a Downlink assignment on the PDCCH addressed to the C-RNTI assigned to the UE on NR Cell 2. | <-- | PDCCH (C-RNTI) | - | - |
| 11 | In the PDCCH occasion when the drx-onDurationTimer is running, the SS indicates the transmission of a DL MAC PDU on the PDCCH of NR Cell 2 with NDI set on the PDCCH. | <-- | MAC PDU | - | - |
| 12 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 11 on NR Cell 2? | --> | HARQ ACK | 1 | P |
| 13 | Check: Does the UE transmit RRCReconfigurationComplete message on NR Cell 2? | --> | RRCReconfigurationComplete | 1 | P |

7.1.1.5.11.3.3	Specific message contents

Table 7.1.1.5.11.3.3-1: RRCReconfiguration (step 1, Table 7.1.1.5.11.3.2-3)

Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 with condition NR_MEAS

Table 7.1.1.5.11.3.3-2: MeasConfig (step 1, Table 7.1.1.5.11.3.2-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR | Table 7.1.1.5.11.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-EventA3 | Table 7.1.1.5.11.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.5.11.3.3-3: MeasObjectNR (Table 7.1.1.5.11.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for SSB of NR Cell 1 |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| nrofSS-BlocksToAverage | Not present |  |  |
| } |  |  |  |

Table 7.1.1.5.11.3.3-4: ReportConfigNR-EventA3 (Table 7.1.1.5.11.3.3-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-142 with condition EVENT_A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | 2 | 1 dB(2*0.5 dB) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportAmount | r1 |  |  |
| maxNrofRS-IndexesToReport | 1 |  |  |
| includeBeamMeasurements | True |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | True |  |  |
| rsrq | False |  |  |
| sinr | False |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.5.11.3.3-5: MeasurementReport (step 4, Table 7.1.1.5.11.3.2-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-5A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| measurementReport SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServingMOList SEQUENCE (SIZE (1..maxNrofServingCells)) OF MeasResultServMO { | 1 entry |  |  |
| MeasResultServMO[1] SEQUENCE { |  | entry 1 |  |
| servCellId | ServCellIndex of NR Cell 1 |  |  |
| measResultServingCell SEQUENCE { |  |  |  |
| physCellId | PCI of NR Cell 1 |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| Sinr | Not present |  |  |
|  | Not checked |  | pc_ss_SINR_Meas |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListNR SEQUENCE (SIZE (1.. maxCellReport)) OF MeasResultNR { | 1 entry |  |  |
| MeasResultNR[1] SEQUENCE { |  | entry 1 |  |
| physCellId | Physical layer cell identity of NR Cell 2 |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| Rsrp | (0..127) |  |  |
| Rsrq | Not present |  |  |
| Sinr | Not present |  |  |
| } |  |  |  |
| resultsCSI-RS-Cell | Not present |  |  |
| } |  |  |  |
| rsIndexResults | Not checked |  |  |
| } |  |  |  |
| cgi-Info | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.5.11.3.3-6: RRCReconfiguration (step 5, Table 7.1.1.5.11.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 with condition LTM |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ltm-Config-r18 CHOICE { |  |  |  |
| setup | LTM-Config-r18 | Table 7.1.1.5.11.3.3-7 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.5.11.3.3-7: LTM-Config-r18 (Table 7.1.1.5.11.3.3-6)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-67D |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-Config-r18 ::= SEQUENCE { |  |  |  |
| ltm-CandidateToAddModList-r18 SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-Candidate-r18 { | 1 entry |  |  |
| LTM-Candidate-r18[1] | LTM-Candidate-r18 | Table 7.1.1.5.11.3.3-8 |  |
| } |  |  |  |
| ltm-CSI-ResourceConfigToAddModList-r18 | Not present |  |  |
| } |  |  |  |

Table 7.1.1.5.11.3.3-8: LTM-Candidate-r18 (step 5, Table 7.1.1.5.11.3.3-7)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-67C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-Candidate-r18 ::= SEQUENCE { |  |  |  |
| ltm-CandidateId-r18 | 1 |  |  |
| ltm-CandidatePCI-r18 | Physical layer cell identity of NR Cell 2 |  |  |
| ltm-CandidateConfig-r18 OCTET STRING (CONTAINING RRCReconfiguration) { |  |  |  |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ltm-TCI-Info-r18 | LTM-TCI-Info specified in TS 38.508-1 [4] Table 4.6.3-67I with condition Joint |  |  |
| } |  |  |  |

Table 7.1.1.5.11.3.3-9: CellGroupConfig (Table 7.1.1.5.11.3.3-8)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with condition PCell change |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| mac-CellGroupConfig SEQUENCE { |  |  |  |
| drx-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| drx-onDurationTimer | ms20 |  |  |
| drx-InactivityTimer | ms10 |  |  |
| drx-HARQ-RTT-TimerDL | 56 | Number of slots=4 due to number of symbol per slot=14 | =0,1,2,3,4 ( 2 with normal CP) |
|  | 48 | Number of slots=4 due to number of symbol per slot=12 | = 2 with external CP |
| drx-HARQ-RTT-TimerUL | 56 | Number of slots=4 due to number of symbol per slot=14 | =0,1,2,3,4 ( 2 with normal CP) |
|  | 48 | Number of slots=4 due to number of symbol per slot=12 | = 2 with external CP |
| drx-RetransmissionTimerDL | sl8 |  |  |
| drx-RetransmissionTimerUL | sl8 |  |  |
| drx-LongCycleStartOffset CHOICE { |  |  |  |
| ms640 | 7 |  |  |
| } |  |  |  |
| shortDRX | Not present |  |  |
| drx-SlotOffset | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 7.1.1.6	Semi-Persistent Scheduling

##### 7.1.1.6.1	Correct handling of DL assignment / Semi-persistent case

7.1.1.6.1.1	Test Purpose (TP)

(1)

with { UE in RRC_Connected state with DRB established and sps-Configuration in DL is enabled }

ensure that {

when { UE receives a DL assignment addressed to its stored CS-RNTI in slot y and with NDI set as 0 and PDCCH content indicates activation }

then { UE starts receiving DL MAC PDU in slots y+n*[semiPersistSchedIntervalDL] where ‘n’ is positive integer starting at zero }

}

(2)

with { UE in RRC_Connected state with DRB established and stored DL SPS assignment to receive MAC PDU in slot y+n*[semiPersistSchedIntervalDL] }

ensure that {

when { UE receives a DL assignment addressed to its CS-RNTI in slot p and with NDI set as 0, where p!= y+n*[semiPersistSchedIntervalDL] }

then { UE starts receiving DL MAC PDU in slots p+n*[semiPersistSchedIntervalDL] and stops receiving DL MAC PDU at slots y+n*[semiPersistSchedIntervalDL]where ‘n’ is positive integer starting at zero }

}

(3)

with { UE in RRC_Connected state with DRB established and stored DL SPS assignment to receive MAC PDU at slot p+n*[semiPersistSchedIntervalDL] }

ensure that {

when { UE receives a DL assignment [for retransmission] addressed to its CS-RNTI in Slot z and with NDI set as 1, where z!= p+n*[semiPersistSchedIntervalDL] }

then { UE receives MAC PDU as per the retransmission grant for CS-RNTI }

}

(4)

with { UE in  RRC_Connected state with DRB established and stored DL SPS assignment to receive MAC PDU at slot y+n*[semiPersistSchedIntervalDL] }

ensure that {

when { UE receives a DL assignment addressed to its C-RNTI in Slot p, such that p= y+n*[semiPersistSchedIntervalDL] }

then { UE receives MAC PDU as per assignment addressed to its C-RNTI }

}

(5)

with { UE in RRC_Connected state with DRB established and stored DL SPS grant to receive MAC PDU at slot z+n*[semiPersistSchedIntervalDL] }

ensure that {

when { UE receives a RRC Message including sps-Configuration with sps-ConfigurationDL set as ‘disable’ and hence resulting in DL SPS grant deactivation }

then { UE deletes the stored sps-Configuration DL parameters and stops receiving DL MAC PDU’s as per stored SPS assignment in slot z+n*[semiPersistSchedIntervalDL] }

}

(6)

with { UE in RRC_Connected state with DRB established and sps-Configuration in DL is enabled }

ensure that {

when { UE receives a DL assignment addressed to its stored CS-RNTI in slot p and with NDI set as 0 and PDCCH content indicates deactivation }

then {UE stops receiving DL MAC PDU’s as per stored SPS assignment }

}

7.1.1.6.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in TS 38.321, clause 5.3.1, 5.8.1 TS 38.300, clause 10.2 and TS 38.213 clause 102. Unless otherwise stated these are Rel-15 requirements.

[TS 38.321, clause 5.3.1]

Downlink assignments received on the PDCCH both indicate that there is a transmission on a DL-SCH for a particular MAC entity and provide the relevant HARQ information.

When the MAC entity has a C-RNTI, Temporary C-RNTI, or CS-RNTI, the MAC entity shall for each PDCCH occasion during which it monitors PDCCH and for each Serving Cell:

1>	if a downlink assignment for this PDCCH occasion and this Serving Cell has been received on the PDCCH for the MAC entity's C-RNTI, or Temporary C-RNTI:

2>	if this is the first downlink assignment for this Temporary C-RNTI:

3>	consider the NDI to have been toggled.

2>	if the downlink assignment is for the MAC entity's C-RNTI, and if the previous downlink assignment indicated to the HARQ entity of the same HARQ process was either a downlink assignment received for the MAC entity's CS-RNTI or a configured downlink assignment:

3>	consider the NDI to have been toggled regardless of the value of the NDI.

2>	indicate the presence of a downlink assignment and deliver the associated HARQ information to the HARQ entity.

1>	else if a downlink assignment for this PDCCH occasion has been received for this Serving Cell on the PDCCH for the MAC entity's CS-RNTI:

2>	if the NDI in the received HARQ information is 1:

3>	consider the NDI for the corresponding HARQ process not to have been toggled;

3>	indicate the presence of a downlink assignment for this Serving Cell and deliver the associated HARQ information to the HARQ entity.

2>	if the NDI in the received HARQ information is 0:

3>	if PDCCH contents indicate SPS deactivation:

4>	clear the configured downlink assignment for this Serving Cell (if any);

4>	if the timeAlignmentTimer, associated with the TAG containing the Serving Cell on which the HARQ feedback is to be transmitted,is running:

5>	indicate a positive acknowledgement for the SPS deactivation to the physical layer.

3>	else if PDCCH content indicates SPS activation:

4>	store the downlink assignment for this Serving Cell and the associated HARQ information as configured downlink assignment;

4>	initialise or re-initialise the configured downlink assignment for this Serving Cell to start in the associated PDSCH duration and to recur according to rules in subclause 5.8.1;

For each Serving Cell and each configured downlink assignment, if configured and activated, the MAC entity shall:

1>	if the PDSCH duration of the configured downlink assignment does not overlap with the PDSCH duration of a downlink assignment received on the PDCCH for this Serving Cell:

2>	instruct the physical layer to receive, in this PDSCH duration, transport block on the DL-SCH according to the configured downlink assignment and to deliver it to the HARQ entity;

2>	set the HARQ Process ID to the HARQ Process ID associated with this PDSCH duration;

2>	consider the NDI bit for the corresponding HARQ process to have been toggled;

2>	indicate the presence of a configured downlink assignment and deliver the stored HARQ information to the HARQ entity.

For configured downlink assignments, the HARQ Process ID associated with the slot where the DL transmission starts is derived from the following equation:

HARQ Process ID = [floor (CURRENT_slot × 10 / (numberOfSlotsPerFrame ×periodicity ))] modulo nrofHARQ-Processes

where CURRENT_slot = [(SFN × numberOfSlotsPerFrame) + slot number in the frame] and numberOfSlotsPerFrame refers to the number of consecutive slots per frame as specified in TS 38.211 [8].

When the MAC entity needs to read BCCH, the MAC entity may, based on the scheduling information from RRC:

1>	if a downlink assignment for this PDCCH occasion has been received on the PDCCH for the SI-RNTI;

2>	indicate a downlink assignment and redundancy version for the dedicated broadcast HARQ process to the HARQ entity.

[TS 38.321, clause 5.8.1]

Semi-Persistent Scheduling (SPS) is configured by RRC per Serving Cell and per BWP. Activation and deactivation of the DL SPS are independent among the Serving Cells.

For the DL SPS, a DL assignment is provided by PDCCH, and stored or cleared based on L1 signalling indicating SPS activation or deactivation.

RRC configures the following parameters when SPS is configured:

-	cs-RNTI: CS-RNTI for activation, deactivation, and retransmission;

-	nrofHARQ-Processes: the number of configured HARQ processes for SPS;

-	periodicity: periodicity of configured downlink assignment for SPS.

When SPS is released by upper layers, all the corresponding configurations shall be released.

After a downlink assignment is configured for SPS, the MAC entity shall consider sequentially that the Nth downlink assignment occurs in the slot for which:

(numberOfSlotsPerFrame × SFN + slot number in the frame) =
[(numberOfSlotsPerFrame × SFNstart time + slotstart time) + N × periodicity × numberOfSlotsPerFrame / 10] modulo (1024 × numberOfSlotsPerFrame)

where SFNstart time and slotstart time are the SFN and slot, respectively, of the first transmission of PDSCH where the configured downlink assignment was (re-)initialised.

[TS 38.300, clause 10.2]

In the downlink, the gNB can dynamically allocate resources to UEs via the C-RNTI on PDCCH(s). A UE always monitors the PDCCH(s) in order to find possible assignments when its downlink reception is enabled (activity governed by DRX when configured). When CA is configured, the same C-RNTI applies to all serving cells.

The gNB may pre-empt an ongoing PDSCH transmission to one UE with a latency-critical transmission to another UE. The gNB can configure UEs to monitor interrupted transmission indications using INT-RNTI on a PDCCH. If a UE receives the interrupted transmission indication, the UE may assume that no useful information to that UE was carried by the resource elements included in the indication, even if some of those resource elements were already scheduled to this UE.

In addition, with Semi-Persistent Scheduling (SPS), the gNB can allocate downlink resources for the initial HARQ transmissions to UEs: RRC defines the periodicity of the configured downlink assignments while PDCCH addressed to CS-RNTI can either signal and activate the configured downlink assignment, or deactivate it; i.e. a PDCCH addressed to CS-RNTI indicates that the downlink assignment can be implicitly reused according to the periodicity defined by RRC, until deactivated.

NOTE:	when required, retransmissions are explicitly scheduled on PDCCH(s).

The dynamically allocated downlink reception overrides the configured downlink assignment in the same serving cell, if they overlap in time. Otherwise a downlink reception according to the configured downlink assignment is assumed, if activated.

When CA is configured, at most one configured downlink assignment can be signalled per serving cell. When BA is configured, at most one configured downlink assignment can be signalled per BWP. On each serving cell, there can be only one configured downlink assignment active at a time, and multiple configured downlink assignment can be simultaneously active on different serving cells only. Activation and deactivation of configured downlink assignments are independent among the serving cells.

[TS 38.213, clause 10.2]

A UE validates, for scheduling activation or scheduling release, a DL SPS assignment PDCCH or configured UL grant Type 2 PDCCH if

-	the CRC of a corresponding DCI format is scrambled with a CS-RNTI provided by cs-RNTI, and

-	the new data indicator field for the enabled transport block is set to '0'.

Validation of the DCI format is achieved if all fields for the DCI format are set according to Table 10.2-1 or Table 10.2-2.

If validation is achieved, the UE considers the information in the DCI format as a valid activation or valid release of DL SPS or configured UL grant Type 2. If validation is not achieved, the UE discards all the information in the DCI format.

Table 10.2-1: Special fields for DL SPS and UL grant Type 2 scheduling activation PDCCH validation

|  | DCI format 0_0/0_1 | DCI format 1_0 | DCI format 1_1 |
| --- | --- | --- | --- |
| HARQ process number | set to all '0's | set to all '0's | set to all '0's |
| Redundancy version | set to '00' | set to '00' | For the enabled transport block: set to '00' |

Table 10.2-2: Special fields for DL SPS and UL grant Type 2 scheduling release PDCCH validation

|  | DCI format 0_0 | DCI format 1_0 |
| --- | --- | --- |
| HARQ process number | set to all '0's | set to all '0's |
| Redundancy version | set to '00' | set to '00' |
| Modulation and coding scheme | set to all '1's | set to all '1's |
| Frequency domain resource assignment | set to all '1's | set to all '1's |

A UE is expected to provide HARQ-ACK information in response to a SPS PDSCH release after N symbols from the last symbol of a PDCCH providing the SPS PDSCH release. If processingType2Enabled of PDSCH-ServingCellConfig is set to enable for the serving cell with the PDCCH providing the SPS PDSCH release,  for ,  for , and  for , otherwise,  for ,  for ,  for , and  for , wherein  corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH providing the SPS PDSCH release and the SCS configuration of a PUCCH carrying the HARQ-ACK information in response to a SPS PDSCH release.

7.1.1.6.1.3	Test description

7.1.1.6.1.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that set to return no data in uplink and that the UM DRB is configured.

7.1.1.6.1.3.2	Test procedure sequence

Table 7.1.1.6.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0A | SS transmits NR RRCReconfiguration message to configure DL SPS. (Note 4) | <-- | (NR RRC: RRCReconfiguration) | - | - |
| 0B | The UE transmits NR RRCReconfigurationComplete message. (Note 5) | --> | (NR RRC: RRCReconfigurationComplete) | - | - |
| 1 | The SS transmits a DL assignment using UE’s CS-RNTI in Slot ‘Y’, NDI=0. | <-- | (DL SPS Grant) | - | - |
| 2 | The SS transmits in Slot ‘Y’, a DL MAC PDU containing a RLC PDU (DL-SQN=0) on UM DRB. | <-- | MAC PDU | - | - |
| 3 | Check: Does the UE transmit a HARQ ACK? | --> | HARQ ACK | 1 | P |
| 4 | The SS transmits in Slot ‘Y+X’, a DL MAC PDU containing a RLC PDU (DL-SQN=1) on DRB. (Note 1) | <-- | MAC PDU | - | - |
| 5 | Check: Does the UE transmit a HARQ ACK? | --> | HARQ ACK | 1 | P |
| 6 | The SS transmits a DL assignment using UE’s CS-RNTI in Slot ‘P’, NDI=0; (Where Y+X<P<Y+2X) | <-- | (DL SPS Grant) | - | - |
| 7 | The SS transmits in Slot ‘P’, a DL MAC PDU containing a RLC PDU (DL-SQN=2) on UM DRB. | <-- | MAC PDU | - | - |
| 8 | Check: Does the UE transmit a HARQ ACK? | --> | HARQ ACK | 2 | P |
| 9 | The SS transmits in Slot ‘Y+2X’, a DL MAC PDU containing a RLC PDU (DL-SQN=3) on UM DRB. | <-- | MAC PDU | - | - |
| 10 | Check: Does the UE transmit a HARQ Feedback? | --> | HARQ ACK/NACK | 2 | F |
| - | EXCEPTION: Steps 11a1-11b2 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that takes place depending on UE support of K0 greater than 0 for PDSCH mapping type A. | - | - | - | - |
| 11a1 | IF pc_dl_SchedulingOffset_PDSCH_TypeA THEN the SS transmits a DL assignment using UE’s C-RNTI in Slot ‘P+X-4’, NDI=0, where TDRA indicates the 2nd entry in pdsch-TimeDomainAllocationList (K0=4), and K1=8. | <-- | (DL Grant) | - | - |
| 11a2 | The SS transmits in Slot ‘P+X’, a DL MAC PDU containing a RLC PDU (DL-SQN=3) on UM DRB. (Note 2) | <-- | MAC PDU | - | - |
| 11a3 | Check: Does the UE transmit a HARQ ACK? | --> | HARQ ACK | 4 | P |
| 11b1 | IF NOT pc_dl_SchedulingOffset_PDSCH_TypeA THEN the SS transmits in Slot ‘P+X’, a DL MAC PDU containing a RLC PDU (DL-SQN=3) on UM DRB. | <-- | MAC PDU | - | - |
| 11b2 | Check: Does the UE transmit a HARQ ACK? | --> | HARQ ACK | 1 | P |
| 12-13 | Void | - | - | - | - |
| 14 | The SS transmits in Slot ‘P+2X’, a DL MAC PDU containing a RLC PDU (DL-SQN=4) on UM DRB. | <-- | MAC PDU | - | - |
| 15 | Check: Does the UE transmit a HARQ ACK? | --> | HARQ ACK | 1 | P |
| 16 | Void | - | - | - | - |
| 17 | The SS transmits in Slot ‘P+3X’, a DL MAC PDU containing 1 RLC PDU’s (DL-SQN=5) on UM DRB; CRC is calculated in such a way will result in CRC error in UE. | <-- | MAC PDU | - | - |
| 18 | Check: Does the UE transmit a HARQ NACK? | --> | HARQ NACK | - | - |
| - | EXCEPTION: Step 19 and 20 shall be repeated until HARQ retransmission count = 3 is reached for MAC PDU at step 17.(Note 3) | - | - | - | - |
| 19 | The SS transmits a DL assignment using UE’s CS-RNTI in Slot ‘Z’, NDI=1; Where (P+3X < Z <P+4X); The DL HARQ process is same as in step 18. | <-- | (DL SPS Grant) | - | - |
| 20 | The SS re-transmits in Slot ‘Z’, a DL MAC PDU containing a RLC PDU (DL-SQN=5) on UM DRB. | <-- | MAC PDU | - | - |
| - | EXCEPTION: Up to 3 HARQ NACK from the UE should be allowed at step 21(Note 3). | - | - | - | - |
| 21 | Check: Does the UE transmit a HARQ ACK? | --> | HARQ ACK | 3 | P |
| 22 | The SS Transmits a PDCCH [for DL SPS deactivation] using UE’s CS-RNTI in slot ‘Q’, NDI=0; Where (P+3X< Q <P+4X). | <-- | PDCCH [for DL SPS explicit release] | - | - |
| 23 | Check: Does the UE transmit a HARQ ACK? | --> | HARQ ACK | 6 | P |
| 24 | The SS transmits in Slot ‘P+5X’, a DL MAC PDU containing 1 RLC PDU’s (DL-SQN=6)on UM DRB; | <-- | MAC PDU | - | - |
| 25 | Check: Does the UE transmit a HARQ Feedback? | --> | HARQ ACK/NACK | 6 | F |
| 26 | The SS Transmits a DL assignment using UE’s CS-RNTI in SF-Num ‘P+6X’, NDI=0 | <-- | (DL SPS Grant) | - | - |
| 27 | The SS transmits in SF-Num ‘P+6X’, a DL MAC PDU containing a RLC PDU (DL-SQN=6)on UM DRB | <-- | MAC PDU | - | - |
| 28 | Check: Does the UE transmit a HARQ ACK? | --> | HARQ ACK | 1 | P |
| 29 | SS transmits NR RRCReconfiguration to disable SPS-ConfigurationDL.(Note 4) | <-- | RRCReconfiguration | - | - |
| 30 | The UE transmits NR RRCReconfigurationComplete.(Note5) | --> | RRCReconfigurationComplete | - | - |
| 30A | The SS transmits a DL assignment using UE’s CS-RNTI in Slot ‘Y’, NDI=0. | <-- | (DL SPS Grant) | - | - |
| 31 | The SS transmits in Slot ‘Y’, a DL MAC PDU containing 1 RLC PDU’s (DL-SQN=7) on UM DRB; | <-- | MAC PDU | - | - |
| 32 | Check: Does the UE transmit a HARQ Feedback? | --> | HARQ ACK/NACK | 5 | F |
| Note 1: X is equal to semiPersistSchedIntervalDL in this document. Note 2: The DL assignment for C-RNTI and hence the size of MAC PDU is different in size than stored CS-RNTI DL assignment in step 6. This assures UE is receiving DSCH data as per DL assignment for C-RNTI and not as per stored grant for CS-RNTI. Note 3: The value 4 for the maximum number of HARQ retransmissions has been chosen based on an assumption that, given the radio conditions used in this test case, a UE soft combiner implementation should have sufficient retransmissions to be able to successfully decode the data in its soft buffer. Note 4: For EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration 36.508 [7], Table 4.6.1-8 using condition EN-DC_EmbedNR_RRCRecon. Note 5: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. Note 6: As per TS 38.508-1[4], the default value for PDSCH slot offset (K0) is 0, hence the DL MAC PDU’s associated with DL SPS grant in Slot X are sent in same slot X. |  |  |  |  |  |

7.1.1.6.1.3.3	Specific message contents

Table 7.1.1.6.1.3.3-1: RRCReconfiguration (Step 0A, Table 7.1.1.6.1.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| secondaryCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC |
|  | Not present |  | NR |
| nonCriticalExtension | Not present |  | EN-DC |
| nonCriticalExtension SEQUENCE{ |  |  | NR |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.6.1.3.3-2: CellGroupConfig (Table 7.1.1.6.1.3.3-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |  |
| servCellIndex |  | 1 |  |  |
|  |  | Not present |  | NR |
| reconfigurationWithSync SEQUENCE { |  |  |  | EN-DC AND pc_dl_SchedulingOffset_PDSCH_TypeA |
| spCellConfigCommon |  | ServingCellConfigCommon |  |  |
| } |  |  |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |  |
| initialDownlinkBWP SEQUENCE { |  |  |  |  |
| sps-Config CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| periodicity |  | ms80 |  |  |
| nrofHARQ-Processes |  | 8 |  |  |
| n1PUCCH-AN SEQUENCE { |  |  |  |  |
| pucch-ResourceId |  | 6 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| physicalCellGroupConfig SEQUENCE { |  |  |  |  |
| cs-RNTI CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| RNTI-Value |  | ‘FFE0’H |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.6.1.3.3-2A: ServingCellConfigCommon (Table 7.1.1.6.1.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-168: |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| downlinkConfigCommon | DownlinkConfigCommon |  |  |
| } |  |  |  |

Table 7.1.1.6.1.3.3-2B: DownlinkConfigCommon (Table 7.1.1.6.1.3.3-2A)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-52: |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DownlinkConfigCommon ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkCommon | Table 7.1.1.6.1.3.3-5C |  |
| } |  |  |  |

Table 7.1.1.6.1.3.3-3: RRCReconfiguration (step 29, Table 7.1.1.6.1.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| secondaryCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC |
|  | Not present |  | NR |
| nonCriticalExtension | Not present |  | EN-DC |
| nonCriticalExtension SEQUENCE { |  |  | NR |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.6.1.3.3-4: CellGroupConfig (Table 7.1.1.6.1.3.3-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |  |
| servCellIndex |  | 1 |  |  |
|  |  | Not present |  | NR |
| spCellConfigDedicated SEQUENCE { |  |  |  |  |
| initialDownlinkBWP SEQUENCE { |  |  |  |  |
| sps-Config CHOICE { |  |  |  |  |
| release |  | Null |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.6.1.3.3-5: SIB1 (Preamble)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| servingCellConfigCommon |  | ServingCellConfigCommonSIB |  | pc_dl_SchedulingOffset_PDSCH_TypeA |
| } |  |  |  |  |

Table 7.1.1.6.1.3.3-5A: ServingCellConfigCommonSIB (Table 7.1.1.6.1.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| downlinkConfigCommon | DownlinkConfigCommonSIB |  |  |
| } |  |  |  |

Table 7.1.1.6.1.3.3-5B: DownlinkConfigCommonSIB (Table 7.1.1.6.1.3.3-5A)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-53 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DownlinkConfigCommonSIB ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkCommon |  |  |
| } |  |  |  |

Table 7.1.1.6.1.3.3-5C: BWP-DownlinkCommon (Tables 7.1.1.6.1.3.3-2B and 7.1.1.6.1.3.3-5B)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-10 with condition InitialBWP_SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkCommon ::= SEQUENCE { |  |  |  |
| pdsch-ConfigCommon CHOICE { |  |  |  |
| setup | PDSCH-ConfigCommon |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.6.1.3.3-5D: PDSCH-ConfigCommon (Table 7.1.1.6.1.3.3-5C)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-101 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| pdsch-TimeDomainAllocationList | PDSCH-TimeDomainResourceAllocationList |  |  |
| } |  |  |  |

Table 7.1.1.6.1.3.3-5E: PDSCH-TimeDomainResourceAllocationList (Table 7.1.1.6.1.3.3-5D)

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-TimeDomainResourceAllocationList ::= SEQUENCE(SIZE(1..maxNrofDL-Allocations)) OF PDSCH-TimeDomainResourceAllocation { | 2 entries |  |  |
| PDSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  | entry 1 |  |
| k0 | 0 |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | Start symbol(S)=2, Length(L)=12 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  | entry 2 |  |
| k0 | 4 |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | S=2, L=12 |  |
| } |  |  |  |
| } |  |  |  |

##### 7.1.1.6.2	Correct handling of UL grant / configured grant Type 1

7.1.1.6.2.1	Test Purpose (TP)

(1)

with { UE in RRC_Connected state with DRB established and sps-Configuration in UL is enabled with Configured grant type 1 }

ensure that {

when { The symbol in which equation [(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] =

(timeDomainOffset × numberOfSymbolsPerSlot + S + N × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)is satisfied }

then { UE starts transmitting UL MAC PDU periodically in the symbol associated with the new re-configured grant }

}

(2)

with { UE in RRC_Connected state with DRB established and configured UL grant type 1 }

ensure that {

when { UE receives a new UL grant type 1 in an RRC message }

then { UE starts transmitting UL MAC PDU periodically in the symbol associated with the new re-configured grant }

}

(3)

with { UE in RRC_Connected state with DRB established and configured UL grant type 1 }

ensure that {

when { UE receives a RRC message including sps-Configuration with rrcConfiguredUplinkGrant set as ‘release’ }

then { UE deletes the stored configured UL Grant type 1 parameters and stops transmitting UL MAC PDU’s as per configured UL grant type 1 }

}

(4)

with { UE in RRC_Connected state with DRB established and configured UL grant type 1 }

ensure that {

when { UE receives a UL grant addressed to its CS-RNTI with NDI set as 1 for retransmission }

then { UE re-transmits MAC PDU as per the new grant }

}

(5)

with { UE in RRC_Connected state with DRB established and configured UL grant type 1 }

ensure that {

when { UE receives a UL grant addressed to its C-RNTI resulting in UL transmission overlap in time domain as configured grante type 1 }

then { UE transmits MAC PDU as per grant addressed to its C-RNTI }

}

7.1.1.6.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 38.321 clauses 5.4.1 and 5.8.2, 3GPP TS 38.300 clause 10.3. Unless otherwise stated these are Rel-15 requirements.

[TS 38.321, clause 5.4.1]

Uplink grant is either received dynamically on the PDCCH, in a Random Access Response, or configured semi-persistently by RRC. The MAC entity shall have an uplink grant to transmit on the UL-SCH. To perform the requested transmissions, the MAC layer receives HARQ information from lower layers.

If the MAC entity has a C-RNTI, a Temporary C-RNTI, or CS-RNTI, the MAC entity shall for each PDCCH occasion and for each Serving Cell belonging to a TAG that has a running timeAlignmentTimer and for each grant received for this PDCCH occasion:

1>	if an uplink grant for this Serving Cell has been received on the PDCCH for the MAC entity's C-RNTI or Temporary C-RNTI; or

1>	if an uplink grant has been received in a Random Access Response:

2>	if the uplink grant is for MAC entity's C-RNTI and if the previous uplink grant delivered to the HARQ entity for the same HARQ process was either an uplink grant received for the MAC entity's CS-RNTI or a configured uplink grant:

3>	consider the NDI to have been toggled for the corresponding HARQ process regardless of the value of the NDI.

2>	if the uplink grant is for MAC entity's C-RNTI, and the identified HARQ process is configured for a configured uplink grant:

3>	start or restart the configuredGrantTimer for the corresponding HARQ process, if configured.

2>	deliver the uplink grant and the associated HARQ information to the HARQ entity.

1>	else if an uplink grant for this PDCCH occasion has been received for this Serving Cell on the PDCCH for the MAC entity's CS-RNTI:

2>	if the NDI in the received HARQ information is 1:

3>	consider the NDI for the corresponding HARQ process not to have been toggled;

3>	start or restart the configuredGrantTimer for the corresponding HARQ process, if configured;

3>	deliver the uplink grant and the associated HARQ information to the HARQ entity.

2>	else if the NDI in the received HARQ information is 0:

3>	if PDCCH contents indicate configured grant Type 2 deactivation:

4>	trigger configured uplink grant confirmation.

3>	else if PDCCH contents indicate configured grant Type 2 activation:

4>	trigger configured uplink grant confirmation;

4>	store the uplink grant for this Serving Cell and the associated HARQ information as configured uplink grant;

4>	initialise or re-initialise the configured uplink grant for this Serving Cell to start in the associated PUSCH duration and to recur according to rules in subclause 5.8.2;

4>	stop the configuredGrantTimer for the corresponding HARQ process, if running;

For each Serving Cell and each configured uplink grant, if configured and activated, the MAC entity shall:

1>	if the PUSCH duration of the configured uplink grant does not overlap with the PUSCH duration of an uplink grant received on the PDCCH or in a Random Access Response for this Serving Cell:

2>	set the HARQ Process ID to the HARQ Process ID associated with this PUSCH duration;

2>	if the configuredGrantTimer for the corresponding HARQ process is not running:

3>	consider the NDI bit for the corresponding HARQ process to have been toggled;

3>	deliver the configured uplink grant and the associated HARQ information to the HARQ entity.

For configured uplink grants, the HARQ Process ID associated with the first symbol of a UL transmission is derived from the following equation:

HARQ Process ID = [floor(CURRENT_symbol/periodicity)] modulo nrofHARQ-Processes

where CURRENT_symbol=(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + slot number in the frame × numberOfSymbolsPerSlot + symbol number in the slot), and numberOfSlotsPerFrame and numberOfSymbolsPerSlot refer to the number of consecutive slots per frame and the number of consecutive symbols per slot, respectively as specified in TS 38.211 [8].

NOTE 1:	CURRENT_symbol refers to the symbol index of the first transmission occasion of a repetition bundle that takes place.

NOTE 2:	A HARQ process is configured for a configured uplink grant if the configured uplink grant is activated and the associated HARQ process ID is less than nrofHARQ-Processes.

NOTE 3:	If the MAC entity receives both a grant in a Random Access Response and an overlapping grant for its C-RNTI or CS-RNTI, requiring concurrent transmissions on the SpCell, the MAC entity may choose to continue with either the grant for its RA-RNTI or the grant for its C-RNTI or CS-RNTI.

[TS 38.321, clause 5.8.2]

There are two types of transmission without dynamic grant:

-	configured grant Type 1 where an uplink grant is provided by RRC, and stored as configured uplink grant;

-	configured grant Type 2 where an uplink grant is provided by PDCCH, and stored or cleared as configured uplink grant based on L1 signalling indicating configured uplink grant activation or deactivation.

Type 1 and Type 2 are configured by RRC per Serving Cell and per BWP. Multiple configurations can be active simultaneously only on different Serving Cells. For Type 2, activation and deactivation are independent among the Serving Cells. For the same Serving Cell, the MAC entity is configured with either Type 1 or Type 2.

RRC configures the following parameters when the configured grant Type 1 is configured:

-	cs-RNTI: CS-RNTI for retransmission;

-	periodicity: periodicity of the configured grant Type 1;

-	timeDomainOffset: Offset of a resource with respect to SFN=0 in time domain;

-	timeDomainAllocation: Allocation of configured uplink grant in time domain which contains startSymbolAndLength (i.e. SLIV in TS 38.214 [7]);

-	nrofHARQ-Processes: the number of HARQ processes for configured grant.

RRC configures the following parameters when the configured grant Type 2 is configured:

-	cs-RNTI: CS-RNTI for activation, deactivation, and retransmission;

-	periodicity: periodicity of the configured grant Type 2;

-	nrofHARQ-Processes: the number of HARQ processes for configured grant.

Upon configuration of a configured grant Type 1 for a Serving Cell by upper layers, the MAC entity shall:

1>	store the uplink grant provided by upper layers as a configured uplink grant for the indicated Serving Cell;

1>	initialise or re-initialise the configured uplink grant to start in the symbol according to timeDomainOffset and S (derived from SLIV as specified in TS 38.214 [7]), and to reoccur with periodicity.

After an uplink grant is configured for a configured grant Type 1, the MAC entity shall consider that the uplink grant recurs associated with each symbol for which:

[(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] =
 (timeDomainOffset × numberOfSymbolsPerSlot + S + N × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot), for all N >= 0.

After an uplink grant is configured for a configured grant Type 2, the MAC entity shall consider that the uplink grant recurs associated with each symbol for which:

[(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] =
[(SFNstart time × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + slotstart time × numberOfSymbolsPerSlot + symbolstart time) + N × periodicity] modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot), for all N >= 0.

where SFNstart time, slotstart time, and symbolstart time are the SFN, slot, and symbol, respectively, of the first transmission opportunity of PUSCH where the configured uplink grant was (re-)initialised.

When a configured uplink grant is released by upper layers, all the corresponding configurations shall be released and all corresponding uplink grants shall be cleared.

The MAC entity shall:

1>	if the configured uplink grant confirmation has been triggered and not cancelled; and

1>	if the MAC entity has UL resources allocated for new transmission:

2>	instruct the Multiplexing and Assembly procedure to generate an Configured Grant Confirmation MAC CE as defined in subclause 6.1.3.7;

2>	cancel the triggered configured uplink grant confirmation.

For a configured grant Type 2, the MAC entity shall clear the configured uplink grant immediately after first transmission of Configured Grant Confirmation MAC CE triggered by the configured uplink grant deactivation.

Retransmissions except for repetition of configured uplink grants use uplink grants addressed to CS-RNTI.

[TS 38.300, clause 10.3]

In the uplink, the gNB can dynamically allocate resources to UEs via the C-RNTI on PDCCH(s). A UE always monitors the PDCCH(s) in order to find possible grants for uplink transmission when its downlink reception is enabled (activity governed by DRX when configured). When CA is configured, the same C-RNTI applies to all serving cells.

In addition, with Configured Grants, the gNB can allocate uplink resources for the initial HARQ transmissions to UEs. Two types of configured uplink grants are defined:

-	With Type 1, RRC directly provides the configured uplink grant (including the periodicity).

-	With Type 2, RRC defines the periodicity of the configured uplink grant while PDCCH addressed to CS-RNTI can either signal and activate the configured uplink grant, or deactivate it; i.e. a PDCCH addressed to CS-RNTI indicates that the uplink grant can be implicitly reused according to the periodicity defined by RRC, until deactivated.

The dynamically allocated uplink transmission overrides the configured uplink grant in the same serving cell, if they overlap in time. Otherwise an uplink transmission according to the configured uplink grant is assumed, if activated.

Retransmissions other than repetitions are explicitly allocated via PDCCH(s).

When CA is configured, at most one configured uplink grant can be signalled per serving cell. When BA is configured, at most one configured uplink grant can be signalled per BWP. On each serving cell, there can be only one configured uplink grant active at a time. A configured uplink grant for one serving cell can either be of Type 1 or Type 2. For Type 2, activation and deactivation of configured uplink grants are independent among the serving cells. When SUL is configured, a configured uplink grant can only be signalled for one of the 2 ULs of the cell.

7.1.1.6.2.3	Test description

7.1.1.6.2.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 and UM DRB should be established on NR Cell 1 and UM DRB/SRB1 are configured according to Table 7.1.1.6.2.3.1-1 if pc_lcp_Restriction = True.

Table 7.1.1.6.2.3.1-1: configuredGrantType1Allowed settings for pc_lcp_Restriction = True

| RB | configuredGrantType1Allowed |
| --- | --- |
| SRB1 | true |
| UM DRB | true |

7.1.1.6.2.3.2	Test procedure sequence

Table 7.1.1.6.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits NR RRCReconfiguration message to configure UL configured grant type 1, timeDomainOffset is set to 9. (Note 1) | <-- | (NR RRC: RRCReconfiguration) | - | - |
| 2 | The UE transmits NR RRCReconfigurationComplete message. (Note 2) | --> | (NR RRC: RRCReconfigurationComplete) | - | - |
| 3 | SS transmits a DL MAC PDU containing 2 RLC SDUs of size 99 bytes in SFN 1022 on UM DRB. (Note 3) | <-- | MAC PDU (two RLC SDUs) | - | - |
| 4 | Check: Does the UE transmit a MAC PDU containing first RLC SDU in Symbol ‘x0’, Slot y0’, SFN ‘z0’ after the SFN in step 3 wraps around? Where [(z0 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (y0 × numberOfSymbolsPerSlot) + x0] = (9 × numberOfSymbolsPerSlot + S + 0 × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot). (Note 4) | --> | MAC PDU (one RLC SDU) | 1 | P |
| 5 | Check: Does the UE transmit a MAC PDU containing second RLC SDU in Symbol ‘x1’, Slot y1’, SFN ‘z1’? Where [(z1 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (y1 × numberOfSymbolsPerSlot) + x1] = (9 × numberOfSymbolsPerSlot + S + 1 × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot). | --> | MAC PDU (one RLC SDU) | 1 | P |
| 6 | SS transmits NR RRCReconfiguration message to configure UL configured grant type 1, timeDomainOffset is set to 19. (Note 1) | <-- | (NR RRC: RRCReconfiguration) | - | - |
| 7 | The UE transmits NR RRCReconfigurationComplete message. (Note 2) | --> | (NR RRC: RRCReconfigurationComplete) | - | - |
| 7A | SS transmits a DL MAC PDU containing 5 RLC SDUs of size 99 bytes in SFN 1022 on UM DRB. (Note 3) | <-- | MAC PDU (five RLC SDUs) | - | - |
| 8 | Check: Does the UE transmit a MAC PDU containing first RLC SDU received in step 7A in Symbol ‘x2’, Slot y2’, SFN ‘z2’? Where [(z2 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (y2 × numberOfSymbolsPerSlot) + x2] = (9 × numberOfSymbolsPerSlot + S + 0 × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot), | --> | MAC PDU (one RLC SDU) | 2 | F |
| 9 | Check: Does the UE transmit a MAC PDU containing first RLC SDU in Symbol ‘x3’, Slot y3’, SFN ‘z3’ after the SFN in step 7A wraps around? Where [(z3 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (y3 × numberOfSymbolsPerSlot) + x3] = (19 × numberOfSymbolsPerSlot + S + 0 × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot). | --> | MAC PDU (one RLC SDU) | 2 | P |
| 10 | Check: Does the UE transmit a MAC PDU containing second RLC SDU in Symbol ‘x4’, Slot y4’, SFN ‘z4’? Where [(z4 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (y4 × numberOfSymbolsPerSlot) + x4] = (19 × numberOfSymbolsPerSlot + S + 1 × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot). | --> | MAC PDU (one RLC SDU) | 2 | P |
| 11 | SS transmits a UL grant addressed to UE’s stored CS-RNTI with NDI set as 1 in Slot ‘p0’of PDCCH (p0 = floor ((y4 +2) * (PDCCHSCS / PUSCHSCS))), allowing the UE to retransmit one loop back SDU. | <-- | (UL Grant) | - | - |
| 12 | Check: Does the UE retransmit the MAC PDU containing the same second RLC SDU as in step 10 in Symbol ‘S’ of Slot ‘q’ of PUSCH? i.e., in the PUSCH slot q = floor (p0 * (PUSCHSCS / PDCCHSCS)) + K2. (Note 5) | --> | MAC PDU (one RLC SDU) | 4 | P |
| 13 | Check: Does the UE transmit a MAC PDU containing third RLC SDU in Symbol ‘x5’, Slot y5’, SFN ‘z5’? Where [(z5 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (y5 × numberOfSymbolsPerSlot) + x5] = (19 × numberOfSymbolsPerSlot + S + 2 × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot). | --> | MAC PDU (one RLC SDU) | 1 | P |
| 14 | SS transmits a UL Grant using UE’s C-RNTI in in Slot ‘p1’ of PDCCH allowing UE to transmit a MAC PDU containing one RLC SDU, where p1 = floor ((z6 × numberOfSlotsPerFrame - K2) * (PDCCHSCS / PUSCHSCS)). (Note 6) Where [(z6 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (y6 × numberOfSymbolsPerSlot) + x6] = (19 × numberOfSymbolsPerSlot + S + 3 × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot). | <-- | (UL Grant) | - | - |
| 15 | Check: Does the UE transmit a MAC PDU containing fourth RLC SDU in Symbol ‘x6’, Slot y6’, SFN ‘z6’? | --> | MAC PDU (one RLC SDU) | 5 | P |
| 16 | Check: Does the UE transmit a MAC PDU containing fifth RLC SDU in Symbol ‘x7’, Slot y7’, SFN ‘z7’? Where [(z7 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (y7 × numberOfSymbolsPerSlot) + x7] = (19 × numberOfSymbolsPerSlot + S + 4 × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot). | --> | MAC PDU (one RLC SDU) | 1 | P |
| 17 | After step 16, SS transmits NR RRCReconfiguration message to release UL configured grant type 1. (Note 1) | <-- | (NR RRC: RRCReconfiguration | - | - |
| 18 | The UE transmits NR RRCReconfigurationComplete message. (Note 2) | --> | (NR RRC: RRCReconfigurationComplete | - | - |
| 19 | SS transmits a DL MAC PDU containing one RLC SDU of size 99 bytes. | <-- | MAC PDU (one RLC SDU) | - | - |
| 20 | Check: Does the UE transmit a MAC PDU containing one RLC SDU in Symbol ‘x8’, Slot y8’, SFN ‘z8’? Where [(z8 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (y8 × numberOfSymbolsPerSlot) + x8] = (19 × numberOfSymbolsPerSlot + S + N × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot). | --> | MAC PDU (one RLC SDU) | 3 | F |
| Note 1: For EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration. Note 2: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. Note 3: According to the setting parameters in Table 7.1.1.6.2.3.3-2, TB size for configured grant type 1 is 848 bits, which is enough to allow the UE to transmit one PDU at a time (99 bytes RLC SDU + 1 byte UM RLC Header + 2 bytes MAC Sub PDU header + 2 bytes for short BSR or padding). Note 4: S is the starting symbol relative to the slot of the first PUSCH transmission for new configured grant type 1. The value of S can be obtained from TS 38.508-1 [4], Table 4.6.3-122. Note 5: q is the slot where the UE shall transmit the PUSCH and is determined by as where is the slot with the scheduling DCI, is based on the numerology of PUSCH. S is the starting symbol relatived to the start of the slot q according to TS 38.214 clause 6.1.2.1. Note 6: The UL grant addressed to C-RNTI should result in UL transmission overlap in time domain as configured grant type 1. |  |  |  |  |  |

7.1.1.6.2.3.3	Specific message contents

Table 7.1.1.6.2.3.3-1: RRCReconfiguration (step 1 and step 6, Table 7.1.1.6.2.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment | Condition |  |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |  |  |
| radioBearerConfig |  | Not present |  |  |  |  |
| secondaryCellGroup |  | CellGroupConfig |  | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC |  |
|  |  | Not Present |  |  | NR |  |
| nonCriticalExtension |  | Not present |  |  |  | EN-DC |
| nonCriticalExtension SEQUENCE{ |  |  |  |  | NR |  |
| masterCellGroup |  | CellGroupConfig |  | OCTET STRING (CONTAINING CellGroupConfig) |  |  |
| dedicatedNAS-MessageList |  | Not present |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |

Table 7.1.1.6.2.3.3-2: CellGroupConfig (Table 7.1.1.6.2.3.3-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList | Not present |  |  |
| mac-CellGroupConfig | Not present |  |  |
| physicalCellGroupConfig SEQUENCE { |  |  |  |
| cs-RNTI CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| RNTI-Value | ‘FFE0’H |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| servCellIndex | Not present |  | NR |
|  | 1 |  | EN-DC |
| reconfigurationWithSync | Not present |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP SEQUENCE { |  |  |  |
| pucch-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| schedulingRequestResourceToAddModList SEQUENCE (SIZE (1..maxNrofSR-Resources)) OF SchedulingRequestResourceConfig { | 1 entry |  |  |
| SchedulingRequestResourceConfig[1] SEQUENCE { |  | entry 1 |  |
| schedulingRequestResourceId | 1 |  |  |
| schedulingRequestID | 0 |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| sl20 | 9 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| configuredGrantConfig CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cg-DMRS-Configuration | DMRS-UplinkConfig | Reference TS 38.508-1[4], Table 4.6.3-51 |  |
| uci-OnPUSCH CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| semiStatic SEQUENCE { | BetaOffsets |  |  |
| betaOffsetACK-Index1 | 9 |  |  |
| betaOffsetACK-Index2 | 9 |  |  |
| betaOffsetACK-Index3 | 9 |  |  |
| betaOffsetCSI-Part1-Index1 | 6 |  |  |
| betaOffsetCSI-Part1-Index2 | 6 |  |  |
| betaOffsetCSI-Part2-Index1 | 6 |  |  |
| betaOffsetCSI-Part2-Index2 | 6 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| resourceAllocation | ResourceAllocationType1 |  |  |
| powerControlLoopToUse | n0 |  |  |
| p0-PUSCH-Alpha | 1 |  |  |
| nrofHARQ-Processes | 16 |  |  |
| repK | n1 |  |  |
| periodicity | Sym40x14 |  | 15kHz |
| periodicity | Sym80x14 |  | 30kHz |
| periodicity | Sym160x14 |  | 60kHz |
| periodicity | Sym320x14 |  | 120kHz |
| rrc-ConfiguredUplinkGrant SEQUENCE { |  |  |  |
| timeDomainOffset | 9 |  | For Step 1 |
|  | 19 |  | For Step 6 |
| timeDomainAllocation | 0 | Reference TS 38.508-1 [4], Table 4.6.3-122 |  |
| frequencyDomainAllocation | BIT STRING (SIZE(18) | BIT STRING (SIZE(18), Equal to NBWPsize * (LRB-1) + RBstart), where LRB = 23 PRB, RBstart = 0, NBWPsize is the size [PRBs] of the active carrier bandwidth part and ontained in TS.38.508-1 [4] clause 4.3.1.1. | FR1_FDD, FR1_TDD |
|  | BIT STRING (SIZE(18) | BIT STRING (SIZE(18), Equal to NBWPsize * (LRB-1) + RBstart), where LRB = 14 PRB, RBstart = 0, NBWPsize is the size [PRBs] of the active carrier bandwidth part and ontained in TS.38.508-1 [4] clause 4.3.1.1. | FR1_FDD AND BW3 |
|  | BIT STRING (SIZE(18) | BIT STRING (SIZE(18), Equal to NBWPsize * (LRB-1) + RBstart), where LRB=23 PRB, RBstart = 0and NBWPsize is the size [PRBs] of the active carrier bandwidth part and contained in TS.38.508-1 [4] clause 4.3.1.2. | FR2_TDD |
| antennaPort | 0 |  |  |
| precodingAndNumberOfLayers | 0 |  |  |
| srs-ResourceIndicator | Not present |  |  |
| mcsAndTBS | 0 |  | FR1_FDD, FR1_TDD |
|  | 0 |  | FR2_TDD |
|  | 2 |  | FR1_FDD AND BW3 |
| pathlossReferenceIndex | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| pusch-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| pusch-TimeDomainAllocationList CHOICE { |  |  |  |
| setup SEQUENCE { | 1 entry |  |  |
| PUSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  | entry 1 |  |
| k2 | n4 |  | FR1 |
|  | n8 |  | FR2 |
| mappingType | typeB |  |  |
| startSymbolAndLength | 0011011 | Start symbol(S)=0, Length(L)=14 | FR1 |
| startSymbolAndLength | 0001110 | S=0, L=2 | FR2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.6.2.3.3-3: RRCReconfiguration (step 17, Table 7.1.1.6.2.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment | Condition |  |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |  |  |
| radioBearerConfig | Not present |  |  |  |  |  |
| secondaryCellGroup | CellGroupConfig |  |  | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC |  |
|  | Not present |  |  |  | NR |  |
| nonCriticalExtension |  | Not present |  |  |  | EN-DC |
| nonCriticalExtension SEQUENCE { |  |  |  |  | NR |  |
| masterCellGroup | CellGroupConfig |  |  | OCTET STRING (CONTAINING CellGroupConfig) |  |  |
| dedicatedNAS-MessageList | Not present |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |

Table 7.1.1.6.2.3.3-4: CellGroupConfig (Table 7.1.1.6.2.3.3-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP SEQUENCE { |  |  |  |
| configuredGrantConfig CHOICE { |  |  |  |
| release | Null |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 7.1.1.6.3	Correct handling of UL grant / configured grant Type 2

7.1.1.6.3.1	Test Purpose (TP)

(1)

with { UE in RRC_Connected state with DRB established and sps-Configuration in UL is enabled }

ensure that {

when { UE receives an UL configured grant type 2 addressed to its stored CS-RNTI with NDI set as 0 and PDCCH content indicates SPS activation }

then { UE starts transmitting UL MAC PDU periodically in the symbol associated with the configured grant }

}

(2)

with { UE in RRC_Connected state with DRB established and configured UL grant type 2 }

ensure that {

when {UE receives a UL grant addressed to its CS-RNTI with NDI set as 0 }

then { UE starts transmitting UL MAC PDU periodically in the symbol associated with the new re-configured grant }

}

(3)

with { UE in RRC_Connected state with DRB established and configured UL grant type 2 }

ensure that {

when { UE receives a UL grant addressed to its CS-RNTI with NDI set as 1 for retransmission }

then { UE re-transmits MAC PDU  as per the new grant }

}

(4)

with{ UE in RRC_Connected state with DRB established and configured UL grant type 2 }

ensure that {

when { UE receives a UL grant addressed to its C-RNTI resulting in UL transmission overlap in time domain as configured grante type 2 }

then { UE transmits MAC PDU as per grant addressed to its C-RNTI }

}

(5)

with { UE in RRC_Connected state with DRB established and configured UL grant type 2 }

ensure that {

when {UE receives a RRC message including sps-Configuration with sps-ConfigurationUL set as ‘disable’ and hence resulting in UL SPS grant deactivation }

then { UE deletes the stored sps-Configuration UL parameters and stops transmitting UL MAC PDU’s as per configured UL grant type 2 }

}

(6)

with{ UE in RRC_Connected state with DRB established and configured UL grant type 2 }

ensure that {

when{ If in the symbol in which UL Configured Grant type 2 is available but the HARQ buffer is empty (no data for transmission) }

then{ UE ignores the UL configured grant type 2 and does not send any MAC PDU }

}

(7)

with { UE in RRC_Connected state with DRB established and sps-Configuration in UL is enabled }

ensure that {

when { UE receives UL configured grant type 2 addressed to its stored CS-RNTI in slot p and with NDI set as 0 and PDCCH content indicates SPS deactivation }

then { UEtransmits configured Grant Confirmation MAC CE confirming the deactivation and  stops transmitting UL MAC PDU’s as per configured UL grant type 2}

}

7.1.1.6.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 38.321 clauses 5.4.1 and 5.8.2, 3GPP TS 38.300 clauses 10.3 and TS 38.213 clause 102. Unless otherwise stated these are Rel-15 requirements.

[TS 38.321, clause 5.4.1]

Uplink grant is either received dynamically on the PDCCH, in a Random Access Response, or configured semi-persistently by RRC. The MAC entity shall have an uplink grant to transmit on the UL-SCH. To perform the requested transmissions, the MAC layer receives HARQ information from lower layers.

If the MAC entity has a C-RNTI, a Temporary C-RNTI, or CS-RNTI, the MAC entity shall for each PDCCH occasion and for each Serving Cell belonging to a TAG that has a running timeAlignmentTimer and for each grant received for this PDCCH occasion:

1>	if an uplink grant for this Serving Cell has been received on the PDCCH for the MAC entity's C-RNTI or Temporary C-RNTI; or

1>	if an uplink grant has been received in a Random Access Response:

2>	if the uplink grant is for MAC entity's C-RNTI and if the previous uplink grant delivered to the HARQ entity for the same HARQ process was either an uplink grant received for the MAC entity's CS-RNTI or a configured uplink grant:

3>	consider the NDI to have been toggled for the corresponding HARQ process regardless of the value of the NDI.

2>	if the uplink grant is for MAC entity's C-RNTI, and the identified HARQ process is configured for a configured uplink grant:

3>	start or restart the configuredGrantTimer for the correponding HARQ process, if configured.

2>	deliver the uplink grant and the associated HARQ information to the HARQ entity.

1>	else if an uplink grant for this PDCCH occasion has been received for this Serving Cell on the PDCCH for the MAC entity's CS-RNTI:

2>	if the NDI in the received HARQ information is 1:

3>	consider the NDI for the corresponding HARQ process not to have been toggled;

3>	start or restart the configuredGrantTimer for the corresponding HARQ process, if configured;

3>	deliver the uplink grant and the associated HARQ information to the HARQ entity.

2>	else if the NDI in the received HARQ information is 0:

3>	if PDCCH contents indicate configured grant Type 2 deactivation:

4>	trigger configured uplink grant confirmation.

3>	else if PDCCH contents indicate configured grant Type 2 activation:

4>	trigger configured uplink grant confirmation;

4>	store the uplink grant for this Serving Cell and the associated HARQ information as configured uplink grant;

4>	initialise or re-initialise the configured uplink grant for this Serving Cell to start in the associated PUSCH duration and to recur according to rules in subclause 5.8.2;

4>	stop the configuredGrantTimer for the corresponding HARQ process, if running;

For each Serving Cell and each configured uplink grant, if configured and activated, the MAC entity shall:

1>	if the PUSCH duration of the configured uplink grant does not overlap with the PUSCH duration of an uplink grant received on the PDCCH or in a Random Access Response for this Serving Cell:

2>	set the HARQ Process ID to the HARQ Process ID associated with this PUSCH duration;

2>	if the configuredGrantTimer for the corresponding HARQ process is not running:

3>	consider the NDI bit for the corresponding HARQ process to have been toggled;

3>	deliver the configured uplink grant and the associated HARQ information to the HARQ entity.

For configured uplink grants, the HARQ Process ID associated with the first symbol of a UL transmission is derived from the following equation:

HARQ Process ID = [floor(CURRENT_symbol/periodicity)] modulo nrofHARQ-Processes

where CURRENT_symbol=(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + slot number in the frame × numberOfSymbolsPerSlot + symbol number in the slot), and numberOfSlotsPerFrame and numberOfSymbolsPerSlot refer to the number of consecutive slots per frame and the number of consecutive symbols per slot, respectively as specified in TS 38.211 [8].

NOTE 1:	CURRENT_symbol refers to the symbol index of the first transmission occasion of a repetition bundle that takes place.

NOTE 2:	A HARQ process is configured for a configured uplink grant if the configured uplink grant is activated and the associated HARQ process ID is less than nrofHARQ-Processes.

NOTE 3:	If the MAC entity receives both a grant in a Random Access Response and an overlapping grant for its C-RNTI or CS-RNTI, requiring concurrent transmissions on the SpCell, the MAC entity may choose to continue with either the grant for its RA-RNTI or the grant for its C-RNTI or CS-RNTI.

[TS 38.321, clause 5.8.2]

There are two types of transmission without dynamic grant:

-	configured grant Type 1 where an uplink grant is provided by RRC, and stored as configured uplink grant;

-	configured grant Type 2 where an uplink grant is provided by PDCCH, and stored or cleared as configured uplink grant based on L1 signalling indicating configured uplink grant activation or deactivation.

Type 1 and Type 2 are configured by RRC per Serving Cell and per BWP. Multiple configurations can be active simultaneously only on different Serving Cells. For Type 2, activation and deactivation are independent among the Serving Cells. For the same Serving Cell, the MAC entity is configured with either Type 1 or Type 2.

RRC configures the following parameters when the configured grant Type 1 is configured:

-	cs-RNTI: CS-RNTI for retransmission;

-	periodicity: periodicity of the configured grant Type 1;

-	timeDomainOffset: Offset of a resource with respect to SFN=0 in time domain;

-	timeDomainAllocation: Allocation of configured uplink grant in time domain which contains startSymbolAndLength (i.e. SLIV in TS 38.214 [7]);

-	nrofHARQ-Processes: the number of HARQ processes for configured grant.

RRC configures the following parameters when the configured grant Type 2 is configured:

-	cs-RNTI: CS-RNTI for activation, deactivation, and retransmission;

-	periodicity: periodicity of the configured grant Type 2;

-	nrofHARQ-Processes: the number of HARQ processes for configured grant.

Upon configuration of a configured grant Type 1 for a Serving Cell by upper layers, the MAC entity shall:

1>	store the uplink grant provided by upper layers as a configured uplink grant for the indicated Serving Cell;

1>	initialise or re-initialise the configured uplink grant to start in the symbol according to timeDomainOffset and S (derived from SLIV as specified in TS 38.214 [7]), and to reoccur with periodicity.

After an uplink grant is configured for a configured grant Type 1, the MAC entity shall consider that the uplink grant recurs associated with each symbol for which:

[(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] =
 (timeDomainOffset × numberOfSymbolsPerSlot + S + N × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot), for all N >= 0.

After an uplink grant is configured for a configured grant Type 2, the MAC entity shall consider that the uplink grant recurs associated with each symbol for which:

[(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] =
[(SFNstart time × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + slotstart time × numberOfSymbolsPerSlot + symbolstart time) + N × periodicity] modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot), for all N >= 0.

where SFNstart time, slotstart time, and symbolstart time are the SFN, slot, and symbol, respectively, of the first transmission opportunity of PUSCH where the configured uplink grant was (re-)initialised.

When a configured uplink grant is released by upper layers, all the corresponding configurations shall be released and all corresponding uplink grants shall be cleared.

The MAC entity shall:

1>	if the configured uplink grant confirmation has been triggered and not cancelled; and

1>	if the MAC entity has UL resources allocated for new transmission:

2>	instruct the Multiplexing and Assembly procedure to generate an Configured Grant Confirmation MAC CE as defined in subclause 6.1.3.7;

2>	cancel the triggered configured uplink grant confirmation.

For a configured grant Type 2, the MAC entity shall clear the configured uplink grant immediately after first transmission of Configured Grant Confirmation MAC CE triggered by the configured uplink grant deactivation.

Retransmissions except for repetition of configured uplink grants use uplink grants addressed to CS-RNTI.

[TS 38.300, clause 10.3]

In the uplink, the gNB can dynamically allocate resources to UEs via the C-RNTI on PDCCH(s). A UE always monitors the PDCCH(s) in order to find possible grants for uplink transmission when its downlink reception is enabled (activity governed by DRX when configured). When CA is configured, the same C-RNTI applies to all serving cells.

In addition, with Configured Grants, the gNB can allocate uplink resources for the initial HARQ transmissions to UEs. Two types of configured uplink grants are defined:

-	With Type 1, RRC directly provides the configured uplink grant (including the periodicity).

-	With Type 2, RRC defines the periodicity of the configured uplink grant while PDCCH addressed to CS-RNTI can either signal and activate the configured uplink grant, or deactivate it; i.e. a PDCCH addressed to CS-RNTI indicates that the uplink grant can be implicitly reused according to the periodicity defined by RRC, until deactivated.

The dynamically allocated uplink transmission overrides the configured uplink grant in the same serving cell, if they overlap in time. Otherwise an uplink transmission according to the configured uplink grant is assumed, if activated.

Retransmissions other than repetitions are explicitly allocated via PDCCH(s).

When CA is configured, at most one configured uplink grant can be signalled per serving cell. When BA is configured, at most one configured uplink grant can be signalled per BWP. On each serving cell, there can be only one configured uplink grant active at a time. A configured uplink grant for one serving cell can either be of Type 1 or Type 2. For Type 2, activation and deactivation of configured uplink grants are independent among the serving cells. When SUL is configured, a configured uplink grant can only be signalled for one of the 2 ULs of the cell.

[TS 38.213, clause 10.2]

A UE validates, for scheduling activation or scheduling release, a DL SPS assignment PDCCH or configured UL grant Type 2 PDCCH if

-	the CRC of a corresponding DCI format is scrambled with a CS-RNTI provided by cs-RNTI, and

-	the new data indicator field for the enabled transport block is set to '0'.

Validation of the DCI format is achieved if all fields for the DCI format are set according to Table 10.2-1 or Table 10.2-2.

If validation is achieved, the UE considers the information in the DCI format as a valid activation or valid release of DL SPS or configured UL grant Type 2. If validation is not achieved, the UE discards all the information in the DCI format.

Table 10.2-1: Special fields for DL SPS and UL grant Type 2 scheduling activation PDCCH validation

|  | DCI format 0_0/0_1 | DCI format 1_0 | DCI format 1_1 |
| --- | --- | --- | --- |
| HARQ process number | set to all '0's | set to all '0's | set to all '0's |
| Redundancy version | set to '00' | set to '00' | For the enabled transport block: set to '00' |

Table 10.2-2: Special fields for DL SPS and UL grant Type 2 scheduling release PDCCH validation

|  | DCI format 0_0 | DCI format 1_0 |
| --- | --- | --- |
| HARQ process number | set to all '0's | set to all '0's |
| Redundancy version | set to '00' | set to '00' |
| Modulation and coding scheme | set to all '1's | set to all '1's |
| Frequency domain resource assignment | set to all '1's | set to all '1's |

A UE is expected to provide HARQ-ACK information in response to a SPS PDSCH release after  N symbols from the last symbol of a PDCCH providing the SPS PDSCH release. If processingType2Enabled of PDSCH-ServingCellConfig is set to enable for the serving cell with the PDCCH providing the SPS PDSCH release,  for ,  for , and  for , otherwise,  for ,  for ,  for , and  for , wherein  corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH providing the SPS PDSCH release and the SCS configuration of a PUCCH carrying the HARQ-ACK information in response to a SPS PDSCH release.

7.1.1.6.3.3	Test description

7.1.1.6.3.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 and UM DRB should be established on NR Cell 1.The loop back size is set to accommodate one RLC SDU in UL of same size as one RLC SDU in DL and 1 byte MAC subheader for Configured Grant Confirmation MAC CE.

7.1.1.6.3.3.2	Test procedure sequence

Table 7.1.1.6.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits NR RRCReconfiguration message to configure UL configured grant type 2. (Note 1) | <-- | (NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits NR RRCReconfigurationComplete message. (Note 2) | --> | (NR RRC: RRCReconfigurationComplete | - | - |
| 3 | SS transmits a DL MAC PDU containing 6 RLC SDUs on UM DRB. | <-- | MAC PDU | - | - |
| 4 | The UE transmits a Scheduling Request, indicating that loop back SDUs are ready for transmission in UL RLC. | --> | (SR) | - | - |
| 5 | SS transmits a UL configured grant type 2 addressed to UE’s stored CS-RNTI in Slot ‘n’ of PDCCH, NDI=0, allowing the UE to transmit one loop back SDU and 1 byte MAC subheader for Configured Grant Confirmation MAC CE. | <-- | (UL configured Grant type 2) | - | - |
| 6 | Check: Does the UE transmit a MAC PDU containing one RLC SDU and a Configured Grant Confirmation MAC CE in Symbol ‘S’ of Slot ‘y’ of PUSCH as per grant in step 5? i.e., in the PUSCH slot y=floor (n * (PUSCHSCS / PDCCHSCS)) + K2. (Note 3) | --> | MAC PDU | 1 | P |
| 7 | Check: Does the UE transmit a MAC PDU containing one RLC SDU in Symbol ‘S’ of Slot ‘y + x’ of PUSCH as per grant in step 5? (Note 4) | --> | MAC PDU | 1 | P |
| 8 | SS transmits a UL configured grant type 2 addressed to UE’s stored CS-RNTI in Slot ‘p’ of PDCCH (p = floor (p0 * (PDCCHSCS / PUSCHSCS))), NDI = 0, allowing the UE to transmit one loop back SDU and 1 byte MAC subheader for Configured Grant Confirmation MAC CE, Where p0 is the slot of PUSCH with y + x < p0 < y + 2x - K2. | <-- | (UL configured Grant type 2) | - | - |
| 9 | Check: Does the UE transmit a MAC PDU containing one RLC SDU and 1 byte MAC subheader for Configured Grant Confirmation MAC CE in Symbol ‘S’ of Slot ‘z’ of PUSCH as per grant in step 8? i.e., in the PUSCH slot z = floor (p * (PUSCHSCS/ PDCCHSCS)) + K2. (Note 3) | --> | MAC PDU | 2 | P |
| 10 | Check: Does the UE transmit a MAC PDU containing one RLC SDU in Symbol ‘S’ of Slot ‘y + 2x’ as per grant in step 5? | --> | MAC PDU | 2 | F |
| 11 | Check: Does the UE transmit a MAC PDU containing one RLC SDU in Symbol ‘S’ of Slot ‘z + x’ of PUSCH as per grant in step 8? | --> | MAC PDU | 2 | P |
| 12 | SS transmits a UL configured grant type 2 addressed to UE’s stored CS-RNTI in Slot ‘q’ of PDCCH (q = floor (q0 * (PDCCHSCS / PUSCHSCS))), NDI = 1; allowing the UE to transmit one loop back SDU. The UL HARQ process is the same as in step 11, Where q0 is the slot of PUSCH with z + x < q0 < z + 2x - K2. | <-- | (UL configured Grant type 2) | - | - |
| 13 | Check: Does the UE transmit a MAC PDU containing the same RLC SDU as in step 11 in Symbol ‘S’ of Slot ‘w’ of PUSCH? i.e., in the PUSCH slot w = floor (q * (PUSCHSCS / PDCCHSCS)) + K2. (Note 3) | --> | MAC PDU | 3 | P |
| 14 | Check: Does the UE transmit a MAC PDU containing one RLC SDU in Symbol ‘S’ of Slot ‘z + 2x’ of PUSCH as per grant in step 8? | --> | MAC PDU | 1 | P |
| 15 | SS transmits a UL Grant using UE’s C-RNTI in in Slot ‘r’ of PDCCH allowing UE to transmit a MAC PDU containing one RLC SDU, where r = floor ((z + 3x - K2) * (PDCCHSCS / PUSCHSCS)). | <-- | (UL Grant) | - | - |
| 16 | Check: Does the UE transmit a MAC PDU containing one RLC SDU in Symbol ‘S’ of Slot ‘z + 3x’ of PUSCH as per grant in step 8? | --> | MAC PDU | 4 | P |
| 17 | Check: Does the UE transmit a MAC PDU in Slot ‘z + 4x’ as per grant in containing zero MAC SDU? (Note 5) | --> | MAC PDU | 6 | F |
| 18 | SS transmits a DL MAC PDU containing 3 RLC SDUs on UM DRB after step 17. | <-- | MAC PDU | - | - |
| 19 | Check: Does the UE transmit a MAC PDU containing one RLC SDU in Symbol ‘S’ of Slot ‘z + 5x’ of PUSCH as per grant in step 8? | --> | MAC PDU | 1 | P |
| 20 | The SS transmits a PDCCH [for UL configured grant type 2 explicit release] using UE’s CS-RNTI in Symbol ‘S’ of slot ‘p’ with NDI=0. Where (z+5x< p <z+6x). | <-- | PDCCH [for UL configured grant type 2 explicit release] | - | - |
| 21 | Check: Does the UE transmit a MAC PDU containing a Configured Grant Confirmation MAC CE and one RLC SDU in ‘S’ of Slot ‘z + 6x’ of PUSCH as per grant in step 8? | --> | MAC PDU | 7 | P |
| 21A | Check: Does the UE transmit a MAC PDU containing one RLC SDU in ‘S’ of Slot ‘z + 7x’ of PUSCH as per grant in step 8? | --> | MAC PDU | 7 | F |
| 22 | SS transmits a UL configured grant type 2 addressed to UE’s stored CS-RNTI in Slot ‘j’ of PDCCH, NDI=0, allowing the UE to transmit one loop back SDU and 1 byte MAC subheader for Configured Grant Confirmation MAC CE. | <-- | (UL configured grant type 2) | - | - |
| 23 | Check: Does the UE transmit a MAC PDU containing one RLC SDU and a Configured Grant Confirmation MAC CE in Symbol ‘S’ of Slot ‘y’ of PUSCH as per grant in step 22? i.e., in the PUSCH slot y=floor (n * (PUSCHSCS / PDCCHSCS)) + K2. (Note 3) | --> | MAC PDU | 1 | P |
| 24 | SS transmits RRCReconfiguration to disable UL configured grant type 2. (Note 1) | <-- | NR RRC: RRCReconfiguration | - | - |
| 25 | The UE transmits RRCReconfigurationComplete. (Note 2) | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 26 | SS transmits a DL MAC PDU containing 1 RLC SDU. | <-- | MAC PDU | - | - |
| 27 | Check: Does the UE transmit a MAC PDU in PUSCH as per configured UL grant type 2 in step 22. | --> | MAC PDU | 5 | F |
| Note 1: For EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration. Note 2: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. Note 3: y is the slot where the UE shall transmit the PUSCH and is determined by as where n is the slot with the scheduling DCI, is based on the numerology of PUSCH. S is the starting symbol relatived to the start of the slot y according to TS 38.214 clause 6.1.2.1. Note 4: x is equal to periodicity / 14 in this test case. Note 5: If the MAC entity does not generate a MAC PDU, one of the conditions which shall be satisfied is that there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.321 clause 5.4.3.1.3. |  |  |  |  |  |

7.1.1.6.3.3.3	Specific message contents

Table 7.1.1.6.3.3.3-1: RRCReconfiguration (step 1, Table 7.1.1.6.3.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | Not present |  |  |
| secondaryCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC |
|  | Not present |  | NR |
| nonCriticalExtension | Not present |  | EN-DC |
| nonCriticalExtension SEQUENCE { |  |  | NR |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| dedicatedNAS-MessageList | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.6.3.3.3-2: CellGroupConfig (Table 7.1.1.6.3.3.3-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList | Not present |  |  |
| mac-CellGroupConfig | Not present |  |  |
| physicalCellGroupConfig SEQUENCE { |  |  |  |
| cs-RNTI CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| RNTI-Value | ‘FFE0’H |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP SEQUENCE { |  |  |  |
| pucch-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| schedulingRequestResourceToAddModList SEQUENCE (SIZE (1..maxNrofSR-Resources)) OF SchedulingRequestResourceConfig { | 1 entry |  |  |
| SchedulingRequestResourceConfig[1] SEQUENCE { |  | entry 1 |  |
| schedulingRequestResourceId | 1 |  |  |
| schedulingRequestID | 0 |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| sl20 | 9 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| configuredGrantConfig CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cg-DMRS-Configuration | DMRS-UplinkConfig | Reference TS 38.508-1 [4], Table 4.6.3-51 |  |
| uci-OnPUSCH CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| semiStatic SEQUENCE { | BetaOffsets |  |  |
| betaOffsetACK-Index1 | 9 |  |  |
| betaOffsetACK-Index2 | 9 |  |  |
| betaOffsetACK-Index3 | 9 |  |  |
| betaOffsetCSI-Part1-Index1 | 6 |  |  |
| betaOffsetCSI-Part1-Index2 | 6 |  |  |
| betaOffsetCSI-Part2-Index1 | 6 |  |  |
| betaOffsetCSI-Part2-Index2 | 6 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| resourceAllocation | ResourceAllocationType1 |  |  |
| powerControlLoopToUse | n0 |  |  |
| p0-PUSCH-Alpha | 0 |  |  |
| nrofHARQ-Processes | 16 |  |  |
| repK | n1 |  |  |
| periodicity | Sym40x14 |  | 15kHz |
| periodicity | Sym80x14 |  | 30kHz |
| periodicity | Sym160x14 |  | 60kHz |
| periodicity | Sym320x14 |  | 120kHz |
| } |  |  |  |
| } |  |  |  |
| pusch-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| pusch-TimeDomainAllocationList CHOICE { |  |  |  |
| setup SEQUENCE { | 1 entry |  |  |
| PUSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  | entry 1 |  |
| k2 | 4 |  | FR1 |
|  | 8 |  | FR2 |
| mappingType | typeB |  |  |
| startSymbolAndLength | 0011011 |  | FR1 |
| startSymbolAndLength | 0001110 |  | FR2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.6.3.3.3-3: RRCReconfiguration (step 24,Table 7.1.1.6.3.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | Not present |  |  |
| secondaryCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC |
|  | Not present |  | NR |
| nonCriticalExtension | Not present |  | EN-DC |
| nonCriticalExtension SEQUENCE { |  |  | NR |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| dedicatedNAS-MessageList | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.6.3.3.3-4: CellGroupConfig (Table 7.1.1.6.3.3.3-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| cellGroupId | 1 |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP SEQUENCE { |  |  |  |
| configuredGrantConfig CHOICE { |  |  |  |
| release | Null |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 7.1.1.6.4	Correct handling of DL assignment / Multi Semi-persistent configuration

7.1.1.6.4.1	Test Purpose (TP)

(1)

with { UE in RRC_Connected state with DRB established and sps-Configuration in DL is enabled }

ensure that {

when { UE receives a DL assignment addressed to its stored CS-RNTI in slot y and with NDI set as 0 with sps-ConfigIndex=0 }

then {UE starts receiving DL MAC PDU in slots y+n*[semiPersistSchedIntervalDL] where ‘n’ is positive integer starting at zero }

}

(2)

with { UE in RRC_Connected state with DRB established and stored DL SPS assignment to receive MAC PDU in slot y+n*[semiPersistSchedIntervalDL] }

ensure that {

when { UE receives another DL assignment addressed to its CS-RNTI in slot p and with NDI set as 0 associated with sps-ConfigIndex=1, where p!= y+n*[semiPersistSchedIntervalDL] }

then { UE starts receiving DL MAC PDU in slots p+n*[semiPersistSchedIntervalDL] and continue receiving DL MAC PDU at slots y+n*[semiPersistSchedIntervalDL] where ‘n’ is positive integer starting at zero }

}

7.1.1.6.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in TS 38.321, clause 5.3.1, 5.8.1 TS 38.300, clause 10.2, and TS 38.213, clause 10.2. Unless otherwise stated these are Rel-16 requirements.

[TS 38.321, clause 5.3.1]

Downlink assignments received on the PDCCH both indicate that there is a transmission on a DL-SCH for a particular MAC entity and provide the relevant HARQ information.

When the MAC entity has a C-RNTI, Temporary C-RNTI, or CS-RNTI, the MAC entity shall for each PDCCH occasion during which it monitors PDCCH and for each Serving Cell:

1>	if a downlink assignment for this PDCCH occasion and this Serving Cell has been received on the PDCCH for the MAC entity's C-RNTI, or Temporary C-RNTI:

2>	if this is the first downlink assignment for this Temporary C-RNTI:

3>	consider the NDI to have been toggled.

2>	if the downlink assignment is for the MAC entity's C-RNTI, and if the previous downlink assignment indicated to the HARQ entity of the same HARQ process was either a downlink assignment received for the MAC entity's CS-RNTI or a configured downlink assignment:

3>	consider the NDI to have been toggled regardless of the value of the NDI.

2>	indicate the presence of a downlink assignment and deliver the associated HARQ information to the HARQ entity.

1>	else if a downlink assignment for this PDCCH occasion has been received for this Serving Cell on the PDCCH for the MAC entity's CS-RNTI:

2>	if the NDI in the received HARQ information is 1:

3>	consider the NDI for the corresponding HARQ process not to have been toggled;

3>	indicate the presence of a downlink assignment for this Serving Cell and deliver the associated HARQ information to the HARQ entity.

2>	if the NDI in the received HARQ information is 0:

3>	if PDCCH contents indicate SPS deactivation:

4>	clear the configured downlink assignment for this Serving Cell (if any);

4>	if the timeAlignmentTimer, associated with the TAG containing the Serving Cell on which the HARQ feedback is to be transmitted, is running:

5>	indicate a positive acknowledgement for the SPS deactivation to the physical layer.

3>	else if PDCCH content indicates SPS activation:

4>	store the downlink assignment for this Serving Cell and the associated HARQ information as configured downlink assignment;

4>	initialise or re-initialise the configured downlink assignment for this Serving Cell to start in the associated PDSCH duration and to recur according to rules in clause 5.8.1;

For each Serving Cell and each configured downlink assignment, if configured and activated, the MAC entity shall:

1>	if the PDSCH duration of the configured downlink assignment does not overlap with the PDSCH duration of a downlink assignment received on the PDCCH for this Serving Cell:

2>	instruct the physical layer to receive, in this PDSCH duration, transport block on the DL-SCH according to the configured downlink assignment and to deliver it to the HARQ entity;

2>	set the HARQ Process ID to the HARQ Process ID associated with this PDSCH duration;

2>	consider the NDI bit for the corresponding HARQ process to have been toggled;

2>	indicate the presence of a configured downlink assignment and deliver the stored HARQ information to the HARQ entity.

For configured downlink assignments without harq-ProcID-Offset, the HARQ Process ID associated with the slot where the DL transmission starts is derived from the following equation:

HARQ Process ID = [floor (CURRENT_slot × 10 / (numberOfSlotsPerFrame × periodicity))] modulo nrofHARQ-Processes

where CURRENT_slot = [(SFN × numberOfSlotsPerFrame) + slot number in the frame] and numberOfSlotsPerFrame refers to the number of consecutive slots per frame as specified in TS 38.211 [8].

NOTE 1:	In case of unaligned SFN across carriers in a cell group, the SFN of the concerned Serving Cell is used to calculate the HARQ Process ID used for configured downlink assignments.

For configured downlink assignments with harq-ProcID-Offset, the HARQ Process ID associated with the slot where the DL transmission starts is derived from the following equation:

HARQ Process ID = [floor (CURRENT_slot × 10 / (numberOfSlotsPerFrame × periodicity))] modulo nrofHARQ-Processes + harq-ProcID-Offset

where CURRENT_slot = [(SFN × numberOfSlotsPerFrame) + slot number in the frame] and numberOfSlotsPerFrame refers to the number of consecutive slots per frame as specified in TS 38.211 [8].

NOTE 2:	CURRENT_slot refers to the slot index of the first transmission occasion of a bundle of configured downlink assignment.

When the MAC entity needs to read BCCH, the MAC entity may, based on the scheduling information from RRC:

1>	if a downlink assignment for this PDCCH occasion has been received on the PDCCH for the SI-RNTI;

2>	indicate a downlink assignment and redundancy version for the dedicated broadcast HARQ process to the HARQ entity.

[TS 38.321, clause 5.8.1]

Semi-Persistent Scheduling (SPS) is configured by RRC per Serving Cell and per BWP. Multiple assignments can be active simultaneously in the same BWP. Activation and deactivation of the DL SPS are independent among the Serving Cells.

For the DL SPS, a DL assignment is provided by PDCCH, and stored or cleared based on L1 signalling indicating SPS activation or deactivation.

RRC configures the following parameters when the SPS is configured:

-	cs-RNTI: CS-RNTI for activation, deactivation, and retransmission;

-	nrofHARQ-Processes: the number of configured HARQ processes for SPS;

-	harq-ProcID-Offset: Offset of HARQ process for SPS;

-	periodicity: periodicity of configured downlink assignment for SPS.

When the SPS is released by upper layers, all the corresponding configurations shall be released.

After a downlink assignment is configured for SPS, the MAC entity shall consider sequentially that the Nth downlink assignment occurs in the slot for which:

(numberOfSlotsPerFrame × SFN + slot number in the frame) =
[(numberOfSlotsPerFrame × SFNstart time + slotstart time) + N × periodicity × numberOfSlotsPerFrame / 10] modulo (1024 × numberOfSlotsPerFrame)

where SFNstart time and slotstart time are the SFN and slot, respectively, of the first transmission of PDSCH where the configured downlink assignment was (re-)initialised.

NOTE:	In case of unaligned SFN across carriers in a cell group, the SFN of the concerned Serving Cell is used to calculate the occurrences of configured downlink assignments.

[TS 38.300, clause 10.2]

In the downlink, the gNB can dynamically allocate resources to UEs via the C-RNTI on PDCCH(s). A UE always monitors the PDCCH(s) in order to find possible assignments when its downlink reception is enabled (activity governed by DRX when configured). When CA is configured, the same C-RNTI applies to all serving cells.

The gNB may pre-empt an ongoing PDSCH transmission to one UE with a latency-critical transmission to another UE. The gNB can configure UEs to monitor interrupted transmission indications using INT-RNTI on a PDCCH. If a UE receives the interrupted transmission indication, the UE may assume that no useful information to that UE was carried by the resource elements included in the indication, even if some of those resource elements were already scheduled to this UE.

In addition, with Semi-Persistent Scheduling (SPS), the gNB can allocate downlink resources for the initial HARQ transmissions to UEs: RRC defines the periodicity of the configured downlink assignments while PDCCH addressed to CS-RNTI can either signal and activate the configured downlink assignment, or deactivate it; i.e. a PDCCH addressed to CS-RNTI indicates that the downlink assignment can be implicitly reused according to the periodicity defined by RRC, until deactivated.

NOTE:	When required, retransmissions are explicitly scheduled on PDCCH(s).

The dynamically allocated downlink reception overrides the configured downlink assignment in the same serving cell, if they overlap in time. Otherwise a downlink reception according to the configured downlink assignment is assumed, if activated.

The UE may be configured with up to 8 active configured downlink assignments for a given BWP of a serving cell. When more than one is configured:

-	The network decides which of these configured downlink assignments are active at a time (including all of them); and

-	Each configured downlink assignment is activated separately using a DCI command and deactivation of configured downlink assignments is done using a DCI command, which can either deactivate a single configured downlink assignment or multiple configured downlink assignments jointly.

[TS 38.213, clause 10.2]

A UE validates, for scheduling activation or scheduling release, a DL SPS assignment PDCCH or a configured UL grant Type 2 PDCCH if

-	the CRC of a corresponding DCI format is scrambled with a CS-RNTI provided by cs-RNTI, and

-	the new data indicator field in the DCI format for the enabled transport block is set to '0', and

-	the DFI flag field, if present, in the DCI format is set to '0', and

-	if validation is for scheduling activation and if the PDSCH-to-HARQ_feedback timing indicator field in the DCI format is present, the PDSCH-to-HARQ_feedback timing indicator field does not provide an inapplicable value from dl-DataToUL-ACK-r16.

If a UE is provided a single configuration for UL grant Type 2 PUSCH or for SPS PDSCH, validation of the DCI format is achieved if all fields for the DCI format are set according to Table 10.2-1 or Table 10.2-2.

If a UE is provided more than one configurations for UL grant Type 2 PUSCH or for SPS PDSCH, a value of the HARQ process number field in a DCI format indicates an activation for a corresponding UL grant Type 2 PUSCH or for a SPS PDSCH configuration with a same value as provided by ConfiguredGrantConfigIndex or by sps-ConfigIndex, respectively. Validation of the DCI format is achieved if the RV field for the DCI format is set as in Table 10.2-3.

If a UE is provided more than one configuration for UL grant Type 2 PUSCH or for SPS PDSCH

-	if the UE is provided ConfiguredGrantConfigType2DeactivationStateList or sps-ConfigDeactivationStateList, a value of the HARQ process number field in a DCI format indicates a corresponding entry for scheduling release of one or more UL grant Type 2 PUSCH or SPS PDSCH configurations

-	if the UE is not provided ConfiguredGrantConfigType2DeactivationStateList or sps-ConfigDeactivationStateList, a value of the HARQ process number field in a DCI format indicates a release for a corresponding UL grant Type 2 PUSCH or for a SPS PDSCH configuration with a same value as provided by ConfiguredGrantConfigIndex or by sps-ConfigIndex, respectively.

Validation of the DCI format is achieved if all fields for the DCI format are set according to Table 10.2-4.

If validation is achieved, the UE considers the information in the DCI format as a valid activation or valid release of DL SPS or configured UL grant Type 2. If validation is not achieved, the UE discards all the information in the DCI format.

Table 10.2-1: Special fields for single DL SPS or single UL grant Type 2 scheduling activation PDCCH validation when a UE is provided a single SPS PDSCH or UL grant Type 2 configuration in the active DL/UL BWP of the scheduled cell

|  | DCI format 0_0/0_1/0_2 | DCI format 1_0/1_2 | DCI format 1_1 |
| --- | --- | --- | --- |
| HARQ process number | set to all '0's | set to all '0's | set to all '0's |
| Redundancy version | set to all '0's | set to all '0's | For the enabled transport block: set to all '0's |

Table 10.2-2: Special fields for single DL SPS or single UL grant Type 2 scheduling release PDCCH validation when a UE is provided a single SPS PDSCH or UL grant Type 2 configuration in the active DL/UL BWP of the scheduled cell

|  | DCI format 0_0/0_1/0_2 | DCI format 1_0/1_1/1_2 |
| --- | --- | --- |
| HARQ process number | set to all '0's | set to all '0's |
| Redundancy version | set to all '0's | set to all '0's |
| Modulation and coding scheme | set to all '1's | set to all '1's |
| Frequency domain resource assignment | set to all '0's for FDRA Type 2 with set to all '1's, otherwise | set to all '0's for FDRA Type 0 or for dynamicSwitch set to all '1's for FDRA Type 1 |

Table 10.2-3: Special fields for a single DL SPS or single UL grant Type 2 scheduling activation PDCCH validation when a UE is provided multiple DL SPS or UL grant Type 2 configurations in the active DL/UL BWP of the scheduled cell

|  | DCI format 0_0/0_1/0_2 | DCI format 1_0/1_2 | DCI format 1_1 |
| --- | --- | --- | --- |
| Redundancy version | set to all '0's | set to all '0's | For the enabled transport block: set to all '0's |

Table 10.2-4: Special fields for a single or multiple DL SPS and UL grant Type 2 scheduling release PDCCH validation when a UE is provided multiple DL SPS or UL grant Type 2 configurations in the active DL/UL BWP of the scheduled cell

|  | DCI format 0_0/0_1/0_2 | DCI format 1_0/1_1/1_2 |
| --- | --- | --- |
| Redundancy version | set to all '0's | set to all '0's |
| Modulation and coding scheme | set to all '1's | set to all '1's |
| Frequency domain resource assignment | set to all '0's for FDRA Type 2 with set to all '1's, otherwise | set to all '0's for FDRA Type 0 or for dynamicSwitch set to all '1's for FDRA Type 1 |

A UE is expected to provide HARQ-ACK information in response to a SPS PDSCH release after  symbols from the last symbol of a PDCCH providing the SPS PDSCH release. If processingType2Enabled of PDSCH-ServingCellConfig is set to enable for the serving cell with the PDCCH providing the SPS PDSCH release,  for ,  for , and  for , otherwise,  for ,  for ,  for , and  for , wherein  corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH providing the SPS PDSCH release and the SCS configuration of a PUCCH carrying the HARQ-ACK information in response to a SPS PDSCH release.

7.1.1.6.4.3	Test description

7.1.1.6.4.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that set to return no data in uplink and that the UM DRB is configured according to Table 7.1.1.6.4.3.1-1.

Table 7.1.1.6.4.3.1-1: RLC parameters

| Uplink UM RLC sn-FieldLength | IF (pc_um_WithShortSN ) size6 ELSE size12 |
| --- | --- |
| Downlink UM RLC sn-FieldLength | F (pc_um_WithShortSN ) size6 ELSE size12 |

7.1.1.6.4.3.2	Test procedure sequence

Table 7.1.1.6.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits NR RRCReconfiguration to config SPS-ConfigurationDL. | <-- | RRCReconfiguration | - | - |
| 2 | The UE transmits NR RRCReconfigurationComplete. | --> | RRCReconfigurationComplete | - | - |
| 3 | The SS transmits a DL assignment using UE’s CS-RNTI associated with sps-ConfigIndex=0 in Slot ‘Y’, NDI=0. | <-- | (DL SPS Grant) | - | - |
| 4 | The SS transmits in Slot ‘Y’, a DL MAC PDU containing a RLC PDU on UM DRB. | <-- | MAC PDU | - | - |
| 5 | Check: Does the UE transmit a HARQ ACK? | --> | HARQ ACK | 1 | P |
| 6 | The SS transmits in Slot ‘Y+X’, a DL MAC PDU containing a RLC PDU on DRB. (Note 1) | <-- | MAC PDU | - | - |
| 7 | Check: Does the UE transmit a HARQ ACK? | --> | HARQ ACK | 1 | P |
| 8 | Void | - | - | - | - |
| 9 | Void | - | - | - | - |
| 10 | The SS transmits a DL assignment using UE’s CS-RNTI associated with sps-ConfigIndex=1 in Slot ‘P’, NDI=0; (Where Y+X<P<Y+2X) | <-- | (DL SPS Grant) | - | - |
| 11 | The SS transmits in Slot ‘P’, a DL MAC PDU containing a RLC PDU on UM DRB. | <-- | MAC PDU | - | - |
| 12 | Check: Does the UE transmit a HARQ ACK? | --> | HARQ ACK | 2 | P |
| 13 | The SS transmits in Slot ‘Y+2X’, a DL MAC PDU containing a RLC PDU on UM DRB. | <-- | MAC PDU | - | - |
| 14 | Check: Does the UE transmit a HARQ Feedback? | --> | HARQ ACK | 2 | P |
| 15 | The SS transmits in Slot ‘P+X’, a DL MAC PDU containing a RLC PDU (DL-SQN=1) on UM DRB. | <-- | MAC PDU | - | - |
| 16 | Check: Does the UE transmit a HARQ Feedback? | --> | HARQ ACK | 2 | P |
| 17 | SS transmits NR RRCReconfiguration to disable SPS-ConfigurationDL. | <-- | RRCReconfiguration | - | - |
| 18 | The UE transmits NR RRCReconfigurationComplete. | --> | RRCReconfigurationComplete | - | - |
| Note 1: X is equal to semiPersistSchedIntervalDL in this document. Note 2: Void. Note 3: Void. Note 4: As per TS 38.508-1[4], the default value for PDSCH slot offset (K0) is 0, hence the DL MAC PDU’s associated with DL SPS grant in Slot X are sent in same slot X. |  |  |  |  |  |

7.1.1.6.4.3.3	Specific message contents

Table 7.1.1.6.4.3.3-1: RRCReconfiguration (step 1, Table 7.1.1.6.4.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| secondaryCellGroup | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.6.4.3.3-2: CellGroupConfig (Table 7.1.1.6.4.3.3-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| physicalCellGroupConfig | PhysicalCellGroupConfig | Table 7.1.1.6.4.3.3-3 |  |
| spCellConfig SEQUENCE { |  |  |  |
| servCellIndex | Not present |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |
| initialDownlinkBWP SEQUENCE { |  |  |  |
| sps-ConfigToAddModList-r16 SEQUENCE (SIZE (1..maxNrofSPS-Config-r16)) OF SPS-Config { | 2 entries |  |  |
| SPS-Config[1] SEQUENCE { |  | entry 1 |  |
| periodicity | ms160 |  |  |
| nrofHARQ-Processes | 8 |  |  |
| sps-ConfigIndex-r16 | 0 |  |  |
| harq-ProcID-Offset-r16 | 0 |  |  |
| sps-ConfigIndex-r16 | 0 |  |  |
| } |  |  |  |
| SPS-Config[2] SEQUENCE { |  | entry 2 |  |
| periodicity | ms160 |  |  |
| nrofHARQ-Processes | 8 |  |  |
| sps-ConfigIndex-r16 | 1 |  |  |
| harq-ProcID-Offset-r16 | 8 |  |  |
| sps-ConfigIndex-r16 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.6.4.3.3-3: PhysicalCellGroupConfig (Table 7.1.1.6.4.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalCellGroupConfig ::= SEQUENCE { |  |  |  |
| cs-RNTI | ‘FFE0’H |  |  |
| } |  |  |  |

Table 7.1.1.6.4.3.3-4: RRCReconfiguration (step 17, Table 7.1.1.6.4.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| secondaryCellGroup | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.6.4.3.3-5: CellGroupConfig (Table 7.1.1.6.4.3.3-4)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |  |
| servCellIndex |  | Not present |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |  |
| initialDownlinkBWP SEQUENCE { |  |  |  |  |
| sps-ConfigToReleaseList-r16 SEQUENCE (SIZE (1..maxNrofSPS-Config-r16)) OF SPS-ConfigIndex-r16 { |  | 2 entries | Release all the SPS entries configured previously |  |
| SPS-ConfigIndex-r16[1] |  | 0 |  |  |
| SPS-ConfigIndex-r16[2] |  | 1 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

##### 7.1.1.6.5	Correct handling of UL grant / Multi configured uplink grants

7.1.1.6.5.1	Test Purpose (TP)

(1)

with { UE in RRC_Connected state with DRB established and sps-Configuration in UL is enabled with Configured grant type 1 }

ensure that {

when { The symbol in which equation [(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] =

(timeDomainOffset × numberOfSymbolsPerSlot + S + N × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) is satisfied }

then { UE starts transmitting UL MAC PDU periodically in the symbol associated with the new re-configured grant }

}

(2)

with { UE in RRC_Connected state with DRB established and configured UL grant type 1 }

ensure that {

when { UE receives another UL grant type 1 in an RRC message with timeDomainOffset 89 }

then { UE starts transmitting UL MAC PDU in symbol with timeDomainOffset 89 and continue transmitting UL MAC PDU in symbol with timeDomainOffset 9 }

}

7.1.1.6.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in TS 38.321, clause 5.4.1, 5.8.2 TS 38.300, clause 10.3. Unless otherwise stated these are Rel-16 requirements.

[TS 38.321, clause 5.4.1]

Uplink grant is either received dynamically on the PDCCH, in a Random Access Response, configured semi-persistently by RRC or determined to be associated with the PUSCH resource of MSGA as specified in clause 5.1.2a. The MAC entity shall have an uplink grant to transmit on the UL-SCH. To perform the requested transmissions, the MAC layer receives HARQ information from lower layers. An uplink grant addressed to CS-RNTI with NDI = 0 is considered as a configured uplink grant. An uplink grant addressed to CS-RNTI with NDI = 1 is considered as a dynamic uplink grant.

If the MAC entity has a C-RNTI, a Temporary C-RNTI, or CS-RNTI, the MAC entity shall for each PDCCH occasion and for each Serving Cell belonging to a TAG that has a running timeAlignmentTimer and for each grant received for this PDCCH occasion:

1>	if an uplink grant for this Serving Cell has been received on the PDCCH for the MAC entity's C-RNTI or Temporary C-RNTI; or

1>	if an uplink grant has been received in a Random Access Response:

2>	if the uplink grant is for MAC entity's C-RNTI and if the previous uplink grant delivered to the HARQ entity for the same HARQ process was either an uplink grant received for the MAC entity's CS-RNTI or a configured uplink grant:

3>	consider the NDI to have been toggled for the corresponding HARQ process regardless of the value of the NDI.

2>	if the uplink grant is for MAC entity's C-RNTI, and the identified HARQ process is configured for a configured uplink grant:

3>	start or restart the configuredGrantTimer for the corresponding HARQ process, if configured.

3>	stop the cg-RetransmissionTimer for the corresponding HARQ process, if running.

2>	deliver the uplink grant and the associated HARQ information to the HARQ entity.

1>	else if an uplink grant for this PDCCH occasion has been received for this Serving Cell on the PDCCH for the MAC entity's CS-RNTI:

2>	if the NDI in the received HARQ information is 1:

3>	consider the NDI for the corresponding HARQ process not to have been toggled;

3>	start or restart the configuredGrantTimer for the corresponding HARQ process, if configured;

3>	stop the cg-RetransmissionTimer for the corresponding HARQ process, if running;

3>	deliver the uplink grant and the associated HARQ information to the HARQ entity.

2>	else if the NDI in the received HARQ information is 0:

3>	if PDCCH contents indicate configured grant Type 2 deactivation:

4>	trigger configured uplink grant confirmation.

3>	else if PDCCH contents indicate configured grant Type 2 activation:

4>	trigger configured uplink grant confirmation;

4>	store the uplink grant for this Serving Cell and the associated HARQ information as configured uplink grant;

4>	initialise or re-initialise the configured uplink grant for this Serving Cell to start in the associated PUSCH duration and to recur according to rules in clause 5.8.2;

4>	stop the configuredGrantTimer for the corresponding HARQ process, if running;

4>	stop the cg-RetransmissionTimer for the corresponding HARQ process, if running.

For each Serving Cell and each configured uplink grant, if configured and activated, the MAC entity shall:

1>	if the MAC entity is configured with lch-basedPrioritization, and the PUSCH duration of the configured uplink grant does not overlap with the PUSCH duration of an uplink grant received in a Random Access Response or with the PUSCH duration of an uplink grant addressed to Temporary C-RNTI or the PUSCH duration of a MSGA payload for this Serving Cell; or

1>	if the MAC entity is not configured with lch-basedPrioritization, and the PUSCH duration of the configured uplink grant does not overlap with the PUSCH duration of an uplink grant received on the PDCCH or in a Random Access Response or the PUSCH duration of a MSGA payload for this Serving Cell:

2>	set the HARQ Process ID to the HARQ Process ID associated with this PUSCH duration;

2>	if, for the corresponding HARQ process, the configuredGrantTimer is not running and cg-RetransmissionTimer is not configured (i.e. new transmission):

3>	consider the NDI bit for the corresponding HARQ process to have been toggled;

3>	deliver the configured uplink grant and the associated HARQ information to the HARQ entity.

2>	else if the cg-RetransmissionTimer for the corresponding HARQ process is configured and not running, then for the corresponding HARQ process:

3>	if the configuredGrantTimer is not running, and the HARQ process is not pending (i.e. new transmission):

4>	consider the NDI bit to have been toggled;

4>	deliver the configured uplink grant and the associated HARQ information to the HARQ entity.

3>	else if the previous uplink grant delivered to the HARQ entity for the same HARQ process was a configured uplink grant (i.e. retransmission on configured grant):

4>	deliver the configured uplink grant and the associated HARQ information to the HARQ entity.

For configured uplink grants neither configured with harq-ProcID-Offset2 nor with cg-RetransmissionTimer, the HARQ Process ID associated with the first symbol of a UL transmission is derived from the following equation:

HARQ Process ID = [floor(CURRENT_symbol/periodicity)] modulo nrofHARQ-Processes

For configured uplink grants with harq-ProcID-Offset2, the HARQ Process ID associated with the first symbol of a UL transmission is derived from the following equation:

HARQ Process ID = [floor(CURRENT_symbol / periodicity)] modulo nrofHARQ-Processes + harq-ProcID-Offset2

where CURRENT_symbol = (SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + slot number in the frame × numberOfSymbolsPerSlot + symbol number in the slot), and numberOfSlotsPerFrame and numberOfSymbolsPerSlot refer to the number of consecutive slots per frame and the number of consecutive symbols per slot, respectively as specified in TS 38.211 [8].

[TS 38.321, clause 5.8.2]There are two types of transmission without dynamic grant:

-	configured grant Type 1 where an uplink grant is provided by RRC, and stored as configured uplink grant;

-	configured grant Type 2 where an uplink grant is provided by PDCCH, and stored or cleared as configured uplink grant based on L1 signalling indicating configured uplink grant activation or deactivation.

Type 1 and Type 2 are configured by RRC for a Serving Cell per BWP. Multiple configurations can be active simultaneously in the same BWP. For Type 2, activation and deactivation are independent among the Serving Cells. For the same BWP, the MAC entity can be configured with both Type 1 and Type 2.

RRC configures the following parameters when the configured grant Type 1 is configured:

-	cs-RNTI: CS-RNTI for retransmission;

-	periodicity: periodicity of the configured grant Type 1;

-	timeDomainOffset: Offset of a resource with respect to SFN = timeReferenceSFN in time domain;

-	timeDomainAllocation: Allocation of configured uplink grant in time domain which contains startSymbolAndLength (i.e. SLIV in TS 38.214 [7]) or startSymbol (i.e. S in TS 38.214 [7]);

-	nrofHARQ-Processes: the number of HARQ processes for configured grant;

-	harq-ProcID-Offset: offset of HARQ process for configured grant for operation with shared spectrum channel access;

-	harq-ProcID-Offset2: offset of HARQ process for configured grant;

-	timeReferenceSFN: SFN used for determination of the offset of a resource in time domain. The UE uses the closest SFN with the indicated number preceding the reception of the configured grant configuration.

RRC configures the following parameters when the configured grant Type 2 is configured:

-	cs-RNTI: CS-RNTI for activation, deactivation, and retransmission;

-	periodicity: periodicity of the configured grant Type 2;

-	nrofHARQ-Processes: the number of HARQ processes for configured grant;

-	harq-ProcID-Offset: offset of HARQ process for configured grant for operation with shared spectrum channel access;

-	harq-ProcID-Offset2: offset of HARQ process for configured grant.

RRC configures the following parameters when retransmissions on configured uplink grant is configured:

-	cg-RetransmissionTimer: the duration after a configured grant (re)transmission of a HARQ process when the UE shall not autonomously retransmit that HARQ process.

Upon configuration of a configured grant Type 1 for a BWP of a Serving Cell by upper layers, the MAC entity shall:

1>	store the uplink grant provided by upper layers as a configured uplink grant for the indicated BWP of the Serving Cell;

1>	initialise or re-initialise the configured uplink grant to start in the symbol according to timeDomainOffset, timeReferenceSFN, and S (derived from SLIV or provided by startSymbol as specified in TS 38.214 [7]), and to reoccur with periodicity.

After an uplink grant is configured for a configured grant Type 1, the MAC entity shall consider sequentially that the Nth (N >= 0) uplink grant occurs in the symbol for which:

[(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] =
 (timeReferenceSFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + timeDomainOffset × numberOfSymbolsPerSlot + S + N × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot).

After an uplink grant is configured for a configured grant Type 2, the MAC entity shall consider sequentially that the Nth (N >= 0) uplink grant occurs in the symbol for which:

[(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] =
[(SFNstart time × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + slotstart time × numberOfSymbolsPerSlot + symbolstart time) + N × periodicity] modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot).

where SFNstart time, slotstart time, and symbolstart time are the SFN, slot, and symbol, respectively, of the first transmission opportunity of PUSCH where the configured uplink grant was (re-)initialised.

If cg-nrofPUSCH-InSlot or cg-nrofSlots is configured for a configured grant Type 1 or Type 2, the MAC entity shall consider the uplink grants occur in those additional PUSCH allocations as specified in clause 6.1.2.3 of TS 38.214 [7].

NOTE:	In case of unaligned SFN across carriers in a cell group, the SFN of the concerned Serving Cell is used to calculate the occurrences of configured uplink grants.

When the configured uplink grant is released by upper layers, all the corresponding configurations shall be released and all corresponding uplink grants shall be cleared.

The MAC entity shall:

1>	if at least one configured uplink grant confirmation has been triggered and not cancelled; and

1>	if the MAC entity has UL resources allocated for new transmission:

2>	if, in this MAC entity, at least one configured uplink grant is configured by configuredGrantConfigToAddModList:

3>	instruct the Multiplexing and Assembly procedure to generate a Multiple Entry Configured Grant Confirmation MAC CE as defined in clause 6.1.3.31.

2>	else:

3>	instruct the Multiplexing and Assembly procedure to generate a Configured Grant Confirmation MAC CE as defined in clause 6.1.3.7.

2>	cancel all triggered configured uplink grant confirmation(s).

For a configured grant Type 2, the MAC entity shall clear the configured uplink grant(s) immediately after first transmission of Configured Grant Confirmation MAC CE or Multiple Entry Configured Grant Confirmation MAC CE which confirms the configured uplink grant deactivation.

Retransmissions use:

-	repetition of configured uplink grants; or

-	received uplink grants addressed to CS-RNTI; or

-	configured uplink grants with cg-RetransmissionTimer configured.

[TS 38.300, clause 10.3]

In the uplink, the gNB can dynamically allocate resources to UEs via the C-RNTI on PDCCH(s). A UE always monitors the PDCCH(s) in order to find possible grants for uplink transmission when its downlink reception is enabled (activity governed by DRX when configured). When CA is configured, the same C-RNTI applies to all serving cells.

The gNB may cancel a PUSCH transmission, or a repetition of a PUSCH transmission, or an SRS transmission of a UE for another UE with a latency-critical transmission. The gNB can configure UEs to monitor cancelled transmission indications using CI-RNTI on a PDCCH. If a UE receives the cancelled transmission indication, the UE shall cancel the PUSCH transmission from the earliest symbol overlapped with the resource or the SRS transmission overlapped with the resource indicated by cancellation (see clause 11.2A of TS 38.213 [38]).

In addition, with Configured Grants, the gNB can allocate uplink resources for the initial HARQ transmissions and HARQ retransmissions to UEs. Two types of configured uplink grants are defined:

-	With Type 1, RRC directly provides the configured uplink grant (including the periodicity).

-	With Type 2, RRC defines the periodicity of the configured uplink grant while PDCCH addressed to CS-RNTI can either signal and activate the configured uplink grant, or deactivate it; i.e. a PDCCH addressed to CS-RNTI indicates that the uplink grant can be implicitly reused according to the periodicity defined by RRC, until deactivated.

If the UE is not configured with enhanced intra-UE overlapping resources prioritization, the dynamically allocated uplink transmission overrides the configured uplink grant in the same serving cell, if they overlap in time. Otherwise an uplink transmission according to the configured uplink grant is assumed, if activated.

If the UE is configured with enhanced intra-UE overlapping resources prioritization, in case a configured uplink grant transmission overlaps in time with dynamically allocated uplink transmission or with another configured uplink grant transmission in the same serving cell, the UE prioritizes the transmission based on the comparison between the highest priority of the logical channels that have data to be transmitted and which are multiplexed or can be multiplexed in MAC PDUs associated with the overlapping resources. Similarly, in case a configured uplink grant transmissions or a dynamically allocated uplink transmission overlaps in time with a scheduling request transmission, the UE prioritizes the transmission based on the comparison between the priority of the logical channel which triggered the scheduling request and the highest priority of the logical channels that have data to be transmitted and which are multiplexed or can be multiplexed in MAC PDU associated with the overlapping resource. In case the MAC PDU associated with a deprioritized transmission has already been generated, the UE keeps it stored to allow the gNB to schedule a retransmission. The UE may also be configured by the gNB to transmit the stored MAC PDU as a new transmission using a subsequent resource of the same configured uplink grant configuration when an explicit retransmission grant is not provided by the gNB.

Retransmissions other than repetitions are explicitly allocated via PDCCH(s) or via configuration of a retransmission timer.

The UE may be configured with up to 12 active configured uplink grants for a given BWP of a serving cell. When more than one is configured, the network decides which of these configured uplink grants are active at a time (including all of them). Each configured uplink grant can either be of Type 1 or Type 2. For Type 2, activation and deactivation of configured uplink grants are independent among the serving cells. When more than one Type 2 configured grant is configured, each configured grant is activated separately using a DCI command and deactivation of Type 2 configured grants is done using a DCI command, which can either deactivate a single configured grant configuration or multiple configured grant configurations jointly.

When SUL is configured, the network should ensure that an active configured uplink grant on SUL does not overlap in time with another active configured uplink grant on the other UL configuration.

For both dynamic grant and configured grant, for a transport block, two or more repetitions can be in one slot, or across slot boundary in consecutive available slots with each repetition in one slot. For both dynamic grant and configured grant Type 2, the number of repetitions can be also dynamically indicated in the L1 signalling. The dynamically indicated number of repetitions shall override the RRC configured number of repetitions, if both are present.

7.1.1.6.5.3	Test description

7.1.1.6.5.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 and UM DRB should be established on NR Cell 1.

7.1.1.6.5.3.2	Test procedure sequence

Table 7.1.1.6.5.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits NR RRCReconfiguration message to configure 2 UL configured grant type 1. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits NR RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | SS transmits a DL MAC PDU containing 6 RLC SDUs of size 96 bytes in SFN 1022 on UM DRB. (Note 1) | <-- | MAC PDU (six RLC SDUs) | - | - |
| 4 | Check: Does the UE transmit a MAC PDU containing first RLC SDU in Symbol ‘x0’, Slot y0’, SFN ‘z0’ after the SFN in step 3 wraps around? Where [(z0 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (y0 × numberOfSymbolsPerSlot) + x0] = (9 × numberOfSymbolsPerSlot + S + 0 × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot). (Note 2) | --> | MAC PDU (one RLC SDU) | 1 | P |
| 5 | Check: Does the UE transmit a MAC PDU containing second RLC SDU in Symbol ‘x1’, Slot y1’, SFN ‘z1’? Where [(z1 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (y1 × numberOfSymbolsPerSlot) + x1] = (89 × numberOfSymbolsPerSlot + S + 0 × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot). | --> | MAC PDU (one RLC SDU) | 2 | P |
| 6 | Check: Does the UE transmit a MAC PDU containing third RLC SDU in Symbol ‘x2’, Slot y2’, SFN ‘z2’? Where [(z2 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (y2 × numberOfSymbolsPerSlot) + x2] = (9 × numberOfSymbolsPerSlot + S + 1 × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot). | --> | MAC PDU (one RLC SDU) | 2 | P |
| 7 | Check: Does the UE transmit a MAC PDU containing fourth RLC SDU in Symbol ‘x3’, Slot y3’, SFN ‘z3'? Where [(z3 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (y3 × numberOfSymbolsPerSlot) + x3] = (89 × numberOfSymbolsPerSlot + S + 1 × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot). | --> | MAC PDU (one RLC SDU) | 2 | P |
| 8 | Check: Does the UE transmit a MAC PDU containing fifth RLC SDU in Symbol ‘x4’, Slot 'y4’, SFN ‘z4’? Where [(z4 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (y4 × numberOfSymbolsPerSlot) + x4] = (9 × numberOfSymbolsPerSlot + S + 2 × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot). | --> | MAC PDU (one RLC SDU) | 2 | P |
| 9 | Check: Does the UE transmit a MAC PDU containing sixth RLC SDU in Symbol ‘x5’, Slot 'y5’, SFN ‘z5’? Where [(z5 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (y5 × numberOfSymbolsPerSlot) + x5] = (89 × numberOfSymbolsPerSlot + S + 2 × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot). | --> | MAC PDU (one RLC SDU) | 2 | P |
| 10 | SS transmits NR RRCReconfiguration message to release configured UL configured grant type 1. | <-- | NR RRC: RRCReconfiguration | - | - |
| 11 | The UE transmits NR RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| Note 1: According to the setting parameters in Table 7.1.1.6.2.3.3-2, TB size for configured grant type 1 is 808 bits, which is enough to allow the UE to transmit one PDU at a time (96 bytes RLC SDU + 1 byte UM RLC Header + 2 bytes MAC Sub PDU header + 2 bytes for short BSR or padding). Note 2: S is the starting symbol relative to the slot of the first PUSCH transmission for new configured grant type 1. The value of S can be obtained from TS 38.508-1 [4], Table 4.6.3-122. Note 3: q is the slot where the UE shall transmit the PUSCH and is determined by as where is the slot with the scheduling DCI, is based on the numerology of PUSCH. S is the starting symbol relative to the start of the slot q according to TS 38.214 clause 6.1.2.1. Note 4: Void. |  |  |  |  |  |

7.1.1.6.5.3.3	Specific message contents

Table 7.1.1.6.5.3.3-1: RRCReconfiguration (step 1, Table 7.1.1.6.5.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | Not present |  |  |
| secondaryCellGroup | Not Present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| dedicatedNAS-MessageList | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.6.5.3.3-2: CellGroupConfig (Table 7.1.1.6.5.3.3-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList | Not present |  |  |
| mac-CellGroupConfig | Not present |  |  |
| physicalCellGroupConfig SEQUENCE { |  |  |  |
| cs-RNTI CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| RNTI-Value | ‘FFE0’H |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| servCellIndex | Not present |  |  |
| reconfigurationWithSync | Not present |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP SEQUENCE { |  |  |  |
| pucch-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| schedulingRequestResourceToAddModList SEQUENCE (SIZE (1..maxNrofSR-Resources)) OF SchedulingRequestResourceConfig { | 1 entry |  |  |
| SchedulingRequestResourceConfig[1] SEQUENCE { |  | entry 1 |  |
| schedulingRequestResourceId | 1 |  |  |
| schedulingRequestID | 0 |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| sl20 | 10 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| configuredGrantConfigToAddModList-r16 SEQUENCE (SIZE (1..maxNrofConfiguredGrantConfig-r16)) OF ConfiguredGrantConfig { | 2 entries |  |  |
| ConfiguredGrantConfig[1] SEQUENCE { |  | entry 1 |  |
| cg-DMRS-Configuration | DMRS-UplinkConfig | Reference TS 38.508-1[4], Table 4.6.3-51 |  |
| uci-OnPUSCH CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| semiStatic SEQUENCE { | BetaOffsets |  |  |
| betaOffsetACK-Index1 | 9 |  |  |
| betaOffsetACK-Index2 | 9 |  |  |
| betaOffsetACK-Index3 | 9 |  |  |
| betaOffsetCSI-Part1-Index1 | 6 |  |  |
| betaOffsetCSI-Part1-Index2 | 6 |  |  |
| betaOffsetCSI-Part2-Index1 | 6 |  |  |
| betaOffsetCSI-Part2-Index2 | 6 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| resourceAllocation | ResourceAllocationType1 |  |  |
| powerControlLoopToUse | n0 |  |  |
| p0-PUSCH-Alpha | 1 |  |  |
| nrofHARQ-Processes | 16 |  |  |
| repK | n1 |  |  |
| periodicity | Sym160x14 |  | 15kHz |
| periodicity | Sym320x14 |  | 30kHz |
| periodicity | Sym640x14 |  | 60kHz |
| periodicity | Sym1280x14 |  | 120kHz |
| rrc-ConfiguredUplinkGrant SEQUENCE { |  |  |  |
| timeDomainOffset | 9 |  |  |
| timeDomainAllocation | 0 | Reference TS 38.508-1 [4], Table 4.6.3-122 |  |
| frequencyDomainAllocation | BIT STRING (SIZE(18) | BIT STRING (SIZE(18), Equal to NBWPsize * (LRB-1) + RBstart), where LRB = 2 PRB, RBstart = 0, NBWPsize is the size [PRBs] of the active carrier bandwidth part and contained in TS.38.508-1 [4] clause 4.3.1.1. | FR1_FDD, FR1_TDD |
| frequencyDomainAllocation | BIT STRING (SIZE(18) | BIT STRING (SIZE(18), Equal to NBWPsize * (LRB-1) + RBstart), where LRB=9 PRB, RBstart = 0and NBWPsize is the size [PRBs] of the active carrier bandwidth part and contained in TS.38.508-1 [4] clause 4.3.1.2. | FR2_TDD |
| antennaPort | 0 |  |  |
| precodingAndNumberOfLayers | 0 |  |  |
| srs-ResourceIndicator | Not present |  |  |
| mcsAndTBS | 18 |  | FR1_FDD, FR1_TDD |
|  | 25 |  | FR2_TDD |
| pathlossReferenceIndex | 0 |  |  |
| } |  |  |  |
| configuredGrantConfigIndex-r16 | 0 |  |  |
| } |  |  |  |
| ConfiguredGrantConfig[2] SEQUENCE { |  | entry 2 |  |
| cg-DMRS-Configuration | DMRS-UplinkConfig | Reference TS 38.508-1[4], Table 4.6.3-51 |  |
| uci-OnPUSCH CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| semiStatic SEQUENCE { | BetaOffsets |  |  |
| betaOffsetACK-Index1 | 9 |  |  |
| betaOffsetACK-Index2 | 9 |  |  |
| betaOffsetACK-Index3 | 9 |  |  |
| betaOffsetCSI-Part1-Index1 | 6 |  |  |
| betaOffsetCSI-Part1-Index2 | 6 |  |  |
| betaOffsetCSI-Part2-Index1 | 6 |  |  |
| betaOffsetCSI-Part2-Index2 | 6 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| resourceAllocation | ResourceAllocationType1 |  |  |
| powerControlLoopToUse | n0 |  |  |
| p0-PUSCH-Alpha | 1 |  |  |
| nrofHARQ-Processes | 16 |  |  |
| repK | n1 |  |  |
| periodicity | Sym160x14 |  | 15kHz |
| periodicity | Sym320x14 |  | 30kHz |
| periodicity | Sym6400x14 |  | 60kHz |
| periodicity | Sym1280x14 |  | 120kHz |
| rrc-ConfiguredUplinkGrant SEQUENCE { |  |  |  |
| timeDomainOffset | 89 |  |  |
| timeDomainAllocation | 0 | Reference TS 38.508-1 [4], Table 4.6.3-122 |  |
| frequencyDomainAllocation | BIT STRING (SIZE(18) | BIT STRING (SIZE(18), Equal to NBWPsize * (LRB-1) + RBstart), where LRB = 2 PRB, RBstart = 0, NBWPsize is the size [PRBs] of the active carrier bandwidth part and contained in TS.38.508-1 [4] clause 4.3.1.1. | FR1_FDD, FR1_TDD |
| frequencyDomainAllocation | BIT STRING (SIZE(18) | BIT STRING (SIZE(18), Equal to NBWPsize * (LRB-1) + RBstart), where LRB=9 PRB, RBstart = 0and NBWPsize is the size [PRBs] of the active carrier bandwidth part and contained in TS.38.508-1 [4] clause 4.3.1.2. | FR2_TDD |
| antennaPort | 0 |  |  |
| precodingAndNumberOfLayers | 0 |  |  |
| srs-ResourceIndicator | Not present |  |  |
| mcsAndTBS | 18 |  | FR1_FDD, FR1_TDD |
|  | 25 |  | FR2_TDD |
| pathlossReferenceIndex | 0 |  |  |
| } |  |  |  |
| configuredGrantConfigIndex-r16 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| pusch-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| pusch-TimeDomainAllocationList CHOICE { |  |  |  |
| setup SEQUENCE { | 1 entry |  |  |
| PUSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  | entry 1 |  |
| k2 | n8 |  | FR1 and FR2 |
| mappingType | typeB |  |  |
| startSymbolAndLength | 0011011 | Start symbol(S)=0, Length(L)=14 | FR1 |
| startSymbolAndLength | 0001110 | S=0, L=2 | FR2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 7.1.1.6.6	Correct handling of UL grant / Multiple CG PUSCH occasions per CG period / Configured grant Type 1

7.1.1.6.6.1	Test Purpose (TP)

(1)

with { UE in RRC_Connected state with DRB established and configured uplink grant is enabled with multi-PUSCH Configured grant type 1 }

ensure that {

when { The symbol in which equation [(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)+ (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] = (timeReferenceSFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + timeDomainOffset × numberOfSymbolsPerSlot + S + N × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)is satisfied }

then { UE transmits UL MAC PDU using the first configured uplink grant in a multi-PUSCH configured grant }

}

(2)

with { UE in RRC_Connected state with DRB established and configured uplink grant is enabled with multi-PUSCH Configured grant type 1 }

ensure that {

when { UE has transmitted UL MAC PDU using the first configured uplink grant in a multi-PUSCH configured grant, and UL Data is still available }

then { UE transmits UL MAC PDU in the Mth (1 < M ≤ nrofSlotsInCG-Period) configured uplink grant within a periodicity occurs (M-1) × numberOfSymbolsPerSlot symbols after the symbol in which the first configured uplink grant in that periodicity occurs }

}

(3)

with { UE in RRC_Connected state with DRB established and configured uplink grant is enabled with multi-PUSCH Configured grant type 1 }

ensure that {

when { UE receives a UL grant addressed to its CS-RNTI with NDI set as 1 for retransmission }

then { UE re-transmits MAC PDU as per the new grant }

}

7.1.1.6.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 38.321 clauses 5.4.1 and 5.8.2, 3GPP TS 38.300 clause 16.15.4.1. Unless otherwise stated these are Rel-18 requirements.

[TS 38.321, clause 5.4.1]

For a multi-PUSCH configured grant (as specified in clause 5.8.2) configured with neither harq-ProcID-Offset2 nor cg-RetransmissionTimer, the HARQ Process ID associated with the first symbol of a UL transmission is derived from the following equation:

HARQ Process ID = [nrofSlotsInCG-Period× floor (CURRENT_symbol / periodicity) + ID_OFFSET] modulo nrofHARQ-Processes

For a multi-PUSCH configured grant configured with harq-ProcID-Offset2, the HARQ Process ID associated with the first symbol of a UL transmission is derived from the following equation:

HARQ Process ID = [nrofSlotsInCG-Period × floor (CURRENT_symbol / periodicity) + ID_OFFSET] modulo nrofHARQ-Processes + harq-ProcID-Offset2

where, if cg-SDT-PeriodicityExt (as defined in TS 38.331 [5]) is not configured, CURRENT_symbol = (SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + slot number in the frame × numberOfSymbolsPerSlot + symbol number in the slot), and numberOfSlotsPerFrame and numberOfSymbolsPerSlot refer to the number of consecutive slots per frame and the number of consecutive symbols per slot, respectively as specified in TS 38.211 [8]; alternatively, if cg-SDT-PeriodicityExt (as defined in TS 38.331 [5]) is configured, CURRENT_symbol = ((H-SFN × numberOfSFNperH-SFN + SFN) × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + slot number in the frame × numberOfSymbolsPerSlot + symbol number in the slot), and numberOfSFNperH-SFN, numberOfSlotsPerFrame and numberOfSymbolsPerSlot refer to the number of consecutive frames per H-SFN, the number of consecutive slots per frame and the number of consecutive symbols per slot, respectively as specified in TS 38.211 [8]. For a multi-PUSCH configured grant, ID_OFFSET equals 0 for the first configured uplink grant within a periodicity of the configuration and K for the Kth (1 ≤ K < nrofSlotsInCG-Period) valid configured uplink grant after the first configured uplink grant within the same periodicity. A configured uplink grant in a multi-PUSCH configured grant is considered valid if it satisfies the conditions specified in clause 6.1 in TS 38.214 [7].

[TS 38.321, clause 5.8.2]

There are two types of transmission without dynamic grant:

-	configured grant Type 1 where an uplink grant is provided by RRC, and stored as configured uplink grant;

-	configured grant Type 2 where an uplink grant is provided by PDCCH, and stored or cleared as configured uplink grant based on L1 signalling indicating configured uplink grant activation or deactivation.

Type 1 and Type 2 are configured by RRC for a Serving Cell per BWP. Multiple configurations can be active simultaneously in the same BWP. For Type 2, activation and deactivation are independent among the Serving Cells. For the same BWP, the MAC entity can be configured with both Type 1 and Type 2.

A multi-PUSCH configured grant has multiple consecutive configured uplink grants within a periodicity. Both Type 1 and Type 2 can be configured for a multi-PUSCH configured grant by RRC.

…

RRC configures the following parameters when the configured grant Type 1 is configured:

-	cs-RNTI: CS-RNTI for retransmission;

-	cg-SDT-CS-RNTI: CS-RNTI for CG-SDT retransmission;

-	cg-SDT-RSRP-ThresholdSSB: an RSRP threshold configured for SSB selection for CG-SDT;

-	cg-RRC-RSRP-ThresholdSSB: an RSRP threshold configured for SSB selection for RACH-less handover;

-	periodicity: periodicity of the configured grant Type 1;

-	timeDomainOffset: Offset of a resource with respect to SFN = timeReferenceSFN in time domain;

-	timeDomainAllocation: Allocation of configured uplink grant in time domain which contains startSymbolAndLength (i.e. SLIV in TS 38.214 [7]) or startSymbol (i.e. S in TS 38.214 [7]);

-	nrofHARQ-Processes: the number of HARQ processes for configured grant;

-	harq-ProcID-Offset: offset of HARQ process for configured grant configured with cg-RetransmissionTimer for operation with shared spectrum channel access;

-	harq-ProcID-Offset2: offset of HARQ process for configured grant not configured with cg-RetransmissionTimer;

-	timeReferenceSFN: SFN used for determination of the offset of a resource in time domain. The UE uses the closest SFN with the indicated number preceding the reception of the configured grant configuration;

…

RRC configures the following parameter when a multi-PUSCH configured grant is configured:

-	nrofSlotsInCG-Period: the number of configured uplink grants in a periodicity of a multi-PUSCH configured grant.

…

Upon configuration of a configured grant Type 1 for a BWP of a Serving Cell by upper layers, the MAC entity shall:

1>	store the uplink grant provided by upper layers as a configured uplink grant for the indicated BWP of the Serving Cell;

1>	initialise or re-initialise the configured uplink grant to start in the symbol according to timeDomainOffset, timeReferenceSFN, and S (derived from SLIV or provided by startSymbol as specified in TS 38.214 [7]), and to reoccur with periodicity.

If cg-SDT-PeriodicityExt (as defined in TS 38.331 [5]) is not configured, after an uplink grant is configured for a configured grant Type 1, the MAC entity shall consider sequentially that the configured uplink grant, or the first configured uplink grant in a multi-PUSCH configured grant, in the Nth (N ≥ 0) periodicity occurs in the symbol for which:

[(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)
	+ (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] =
	(timeReferenceSFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot
	+ timeDomainOffset × numberOfSymbolsPerSlot + S + N × periodicity)
	modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)

…

For a multi-PUSCH configured grant Type 1, the Mth (1 < M ≤ nrofSlotsInCG-Period) configured uplink grant within a periodicity occurs (M-1) × numberOfSymbolsPerSlot symbols after the symbol in which the first configured uplink grant in that periodicity occurs.

…

Retransmissions use:

-	repetition of configured uplink grants; or

-	received uplink grants addressed to CS-RNTI; or

-	configured uplink grants with cg-RetransmissionTimer, cg-RRC-RetransmissionTimer or cg-SDT-RetransmissionTimer configured.

[TS 38.321, clause 16.15.4.1]

The following enhancements for configured grant-based PUSCH transmission are introduced:

-	Support of multiple CG PUSCH transmission occasions within a single period of a CG configuration;

-	Indication of unused CG PUSCH occasion(s) of a CG configuration with Uplink Control Information multiplexed in CG PUSCH transmission of the CG configuration.

7.1.1.6.6.3	Test description

7.1.1.6.6.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 and UM DRB should be established on NR Cell 1 and UM DRB/SRB1 are configured according to Table 7.1.1.6.6.3.1-1 if pc_lcp_Restriction = True.

Table 7.1.1.6.6.3.1-1: configuredGrantType1Allowed settings for pc_lcp_Restriction = True

| RB | configuredGrantType1Allowed |
| --- | --- |
| SRB1 | true |
| UM DRB | true |

7.1.1.6.6.3.2	Test procedure sequence

Table 7.1.1.6.6.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits NR RRCReconfiguration messageto configure UL configured grant type 1 in SFN 900. | <-- | (NR RRC: RRCReconfiguration) | - | - |
| 2 | The UE transmits NR RRCReconfigurationComplete message. | --> | (NR RRC: RRCReconfigurationComplete) | - | - |
| 3 | SS transmits a DL MAC PDU containing 4 RLC SDUs (RLC SDU #1, RLC SDU #2, RLC SDU #3 and RLC SDU #4) in SFN 1021 on UM DRB. The size of each RLC SDU is 99 bytes (Note 1) | <-- | MAC PDU (four RLC SDUs) | - | - |
| 4 | Check: Does the UE transmit a MAC PDU containing RLC SDU #1 received in step 3 in Symbol 0-13, Slot ‘x1’, and SFN 0 after the SFN in step 3 wraps around? (Note 2) | --> | MAC PDU (RLC SDU #1) | 1 | P |
| 5 | Check: Does the UE transmit a MAC PDU containing RLC SDU #2 received in step 3 in Symbol 0-13, Slot ‘x2’, and SFN 0? (Note 2) | --> | MAC PDU (RLC SDU #2) | 2 | P |
| 6 | Check: Does the UE transmit a MAC PDU containing RLC SDU #3 received in step 3 in Symbol 0-13, Slot ‘x1’, and SFN 4? (Note 2) | --> | MAC PDU (RLC SDU #3) | 1 | P |
| 7 | Check: Does the UE transmit a MAC PDU containing RLC SDU #4 received in step 3 in Symbol 0-13, Slot ‘x2’, and SFN 4? (Note 2) | --> | MAC PDU (RLC SDU #4) | 2 | P |
| 8 | SS transmits a UL grant addressed to UE’s stored CS-RNTI with NDI set as 1 and HARQ process id set to be the same as step 9 in SFN 6 , allowing the UE to retransmit one loop back SDU.(Note 3) | <-- | (UL Grant) | - | - |
| 9 | Check: Does the UE retransmit the MAC PDU containing RLC SDU #4 as in step 7 | --> | MAC PDU (RLC SDU #4) | 3 | P |
| 10 | The SS transmits NR RRCReconfiguration message to release UL configured grant type 1. | <-- | (NR RRC: RRCReconfiguration | - | - |
| 11 | The UE transmits NR RRCReconfigurationComplete message. | --> | (NR RRC: RRCReconfigurationComplete | - | - |
| Note 1 According to the setting parameters in Table 7.1.1.6.6.3.3-2, TB size for configured grant type 1 is 848 bits, which is enough to allow the UE to transmit one PDU at a time (99 bytes RLC SDU + 1 byte UM RLC Header + 2 bytes MAC Sub PDU header + 2 bytes for short BSR or padding). Note 2: For (TDD AND SCS15) OR SCS120, x1=4 and x2 =9; For other condition, x1=8 and x2=9. Note 3: Harq process id is calculated based on the formula “HARQ Process ID = [nrofSlotsInCG-Period× floor (CURRENT_symbol / periodicity) + ID_OFFSET] modulo nrofHARQ-Processes” specified in TS 38.321 5.4.1. |  |  |  |  |  |

7.1.1.6.6.3.3	Specific message contents

Table 7.1.1.6.6.3.3-1: RRCReconfiguration (step 1, Table 7.1.1.6.6.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | Not present |  |  |
| secondaryCellGroup | Not Present |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| dedicatedNAS-MessageList | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.6.6.3.3-2: CellGroupConfig (Table 7.1.1.6.6.3.3-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList | Not present |  |  |
| mac-CellGroupConfig | Not present |  |  |
| physicalCellGroupConfig SEQUENCE { |  |  |  |
| cs-RNTI CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| RNTI-Value | ‘FFE0’H |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync | Not present |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP SEQUENCE { |  |  |  |
| configuredGrantConfig | ConfiguredGrantConfig | Table 7.1.1.6.6.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.6.6.3.3-3: ConfiguredGrantConfig (Table 7.1.1.6.6.3.3-2)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-26 with condition CG_Config_Type1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConfiguredGrantConfig ::= SEQUENCE { |  |  |  |
| uci-OnPUSCH CHOICE { |  |  |  |
| setup CHOICE { |  |  |  |
| semiStatic | BetaOffsets | TS 38.508-1[4], Table 4.6.3-6A |  |
| } |  |  |  |
| } |  |  |  |
| rrc-ConfiguredUplinkGrant SEQUENCE { |  |  |  |
| timeDomainOffset | 4 | Valid CG PUSCH occasion are slot 4 and slot 9. | (TDD AND SCS15) OR SCS120 |
|  | 8 | Valid CG PUSCH occasion are slot 8 and slot 9. |  |
| timeDomainAllocation | 0 |  |  |
| frequencyDomainAllocation | BIT STRING (SIZE(18) | Equals to NBWPsize * (NBWPsize – LRB + 1) + (NBWPsize -1 - RBstart), where LRB = 23 PRB, RBstart = 0, NBWPsize is the size [PRBs] of the carrier bandwidth and contained in 38.508-1 [4] clause 6.2.3.1 | FR1 |
|  | BIT STRING (SIZE(18) | Equals to NBWPsize * (NBWPsize – LRB + 1) + (NBWPsize -1 - RBstart), where LRB = 14 PRB, RBstart = 0, NBWPsize is the size [PRBs] of the carrier bandwidth and contained in 38.508-1 [4] clause 6.2.3.1 | FR1 AND BW3 |
|  | BIT STRING (SIZE(18) | Equal to NBWPsize * (LRB-1) + RBstart), where LRB = 23 PRB, RBstart = 0, NBWPsize is the size [PRBs] of the carrier bandwidth and contained in 38.508-1 [4] clause 6.2.3.1 | FR2 |
| mcsAndTBS | 0 |  |  |
|  | 2 |  | FR1_FDD AND BW3 |
| } |  |  |  |
| cg-RetransmissionTimer-r16 | Not present |  |  |
| cg-minDFI-Delay-r16 | Not present |  |  |
| cg-nrofPUSCH-InSlot-r16 | Not present |  |  |
| cg-nrofSlots-r16 | Not present |  |  |
| cg-UCI-Multiplexing-r16 | Not present |  |  |
| cg-COT-SharingOffset-r16 | Not present |  |  |
| betaOffsetCG-UCI-r16 | Not present |  |  |
| cg-COT-SharingList-r16 | Not present |  |  |
| harq-ProcID-Offset-r16 | Not present |  |  |
| harq-ProcID-Offset2-r16 | Not present |  |  |
| configuredGrantConfigIndex-r16 | Not present |  |  |
| configuredGrantConfigIndexMAC-r16 | Not present |  |  |
| periodicityExt-r16 | Not present |  |  |
| startingFromRV0-r16 | Not present |  |  |
| phy-PriorityIndex-r16 | Not present |  |  |
| autonomousTx-r16 | Not present |  |  |
| cg-betaOffsetsCrossPri0-r17 | Not present |  |  |
| cg-betaOffsetsCrossPri1-r17 | Not present |  |  |
| mappingPattern-r17 | Not present |  |  |
| sequenceOffsetForRV-r17 | Not present |  |  |
| p0-PUSCH-Alpha2-r17 | Not present |  |  |
| powerControlLoopToUse2-r17 | Not present |  |  |
| cg-COT-SharingList-r17 | Not present |  |  |
| periodicityExt-r17 | Not present |  |  |
| repK-v1710 | Not present |  |  |
| nrofHARQ-Processes-v1700 | Not present |  |  |
| harq-ProcID-Offset2-v1700 | Not present |  |  |
| configuredGrantTimer-v1700 | Not present |  |  |
| cg-minDFI-Delay-v1710 | Not present |  |  |
| harq-ProcID-Offset-v1730 | Not present |  |  |
| cg-nrofSlots-r17 | Not present |  |  |
| disableCG-RetransmissionMonitoring-r18 | Not present |  |  |
| nrofSlotsInCG-Period-r18 | 6 |  | (TDD AND SCS15) OR SCS120 |
|  | 2 |  |  |
| uto-UCI-Config-r18 | Not present |  |  |
| } |  |  |  |

Table 7.1.1.6.6.3.3-4: RRCReconfiguration (step 10, Table 7.1.1.6.6.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | Not present |  |  |
| secondaryCellGroup | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  | NR |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| dedicatedNAS-MessageList | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.6.6.3.3-5: CellGroupConfig (Table 7.1.1.6.6.3.3-4)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP SEQUENCE { |  |  |  |
| configuredGrantConfig CHOICE { |  |  |  |
| release | Null |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 7.1.1.6.7	Correct handling of UL grant / Multiple CG PUSCH occasions per CG period / Configured grant Type 2

7.1.1.6.7.1	Test Purpose (TP)

(1)

with { UE in RRC_Connected state with DRB established and configured uplink grant is enabled with multi-PUSCH Configured grant type 2 }

ensure that {

when { UE receives an UL configured grant type 2 addressed to its stored CS-RNTI with NDI set as 0 and PDCCH content indicates single UL grant Type 2 scheduling activation }

then { UE transmits UL MAC PDU using the first transmission opportunity of PUSCH where the configured uplink grant was (re-)initialised}

}

(2)

with { UE in RRC_Connected state with DRB established and multi-PUSCH Configured grant type 2 is activated}

ensure that {

when { The symbol in which equation [(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)+ (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] =[(SFNstart time × numberOfSlotsPerFrame × numberOfSymbolsPerSlot+ slotstart time × numberOfSymbolsPerSlot + symbolstart time) + N × periodicity] modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)is satisfied }

then { UE transmits UL MAC PDU using the first configured uplink grant in a multi-PUSCH configured grant }

}

(3)

with { UE in RRC_Connected state with DRB established and multi-PUSCH Configured grant type 2 is activated }

ensure that {

when { UE has transmitted UL MAC PDU using the first configured uplink grant in a multi-PUSCH configured grant or first transmission opportunity of PUSCH where the configured uplink grant was (re-)initialised, and UL Data is still available }

then { UE transmits UL MAC PDU in the Mth (1 < M ≤ nrofSlotsInCG-Period) configured uplink grant within a periodicity occurs (M-1) × numberOfSymbolsPerSlot symbols after the symbol in which the first configured uplink grant in that periodicity occurs }

}

(4)

with { UE in RRC_Connected state with DRB established and multi-PUSCH Configured grant type 2 is activated }

ensure that {

when { UE receives a UL grant addressed to its CS-RNTI with NDI set as 1 for retransmission }

then { UE re-transmits MAC PDU as per the new grant }

}

7.1.1.6.7.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 38.321 clauses 5.4.1 and 5.8.2, 3GPP TS 38.300 clause 16.15.4.1. Unless otherwise stated these are Rel-18 requirements.

[TS 38.321, clause 5.4.1]

For a multi-PUSCH configured grant (as specified in clause 5.8.2) configured with neither harq-ProcID-Offset2 nor cg-RetransmissionTimer, the HARQ Process ID associated with the first symbol of a UL transmission is derived from the following equation:

HARQ Process ID = [nrofSlotsInCG-Period× floor (CURRENT_symbol / periodicity) + ID_OFFSET] modulo nrofHARQ-Processes

For a multi-PUSCH configured grant configured with harq-ProcID-Offset2, the HARQ Process ID associated with the first symbol of a UL transmission is derived from the following equation:

HARQ Process ID = [nrofSlotsInCG-Period × floor (CURRENT_symbol / periodicity) + ID_OFFSET] modulo nrofHARQ-Processes + harq-ProcID-Offset2

where, if cg-SDT-PeriodicityExt (as defined in TS 38.331 [5]) is not configured, CURRENT_symbol = (SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + slot number in the frame × numberOfSymbolsPerSlot + symbol number in the slot), and numberOfSlotsPerFrame and numberOfSymbolsPerSlot refer to the number of consecutive slots per frame and the number of consecutive symbols per slot, respectively as specified in TS 38.211 [8]; alternatively, if cg-SDT-PeriodicityExt (as defined in TS 38.331 [5]) is configured, CURRENT_symbol = ((H-SFN × numberOfSFNperH-SFN + SFN) × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + slot number in the frame × numberOfSymbolsPerSlot + symbol number in the slot), and numberOfSFNperH-SFN, numberOfSlotsPerFrame and numberOfSymbolsPerSlot refer to the number of consecutive frames per H-SFN, the number of consecutive slots per frame and the number of consecutive symbols per slot, respectively as specified in TS 38.211 [8]. For a multi-PUSCH configured grant, ID_OFFSET equals 0 for the first configured uplink grant within a periodicity of the configuration and K for the Kth (1 ≤ K < nrofSlotsInCG-Period) valid configured uplink grant after the first configured uplink grant within the same periodicity. A configured uplink grant in a multi-PUSCH configured grant is considered valid if it satisfies the conditions specified in clause 6.1 in TS 38.214 [7].

[TS 38.321, clause 5.8.2]

There are two types of transmission without dynamic grant:

-	configured grant Type 1 where an uplink grant is provided by RRC, and stored as configured uplink grant;

-	configured grant Type 2 where an uplink grant is provided by PDCCH, and stored or cleared as configured uplink grant based on L1 signalling indicating configured uplink grant activation or deactivation.

Type 1 and Type 2 are configured by RRC for a Serving Cell per BWP. Multiple configurations can be active simultaneously in the same BWP. For Type 2, activation and deactivation are independent among the Serving Cells. For the same BWP, the MAC entity can be configured with both Type 1 and Type 2.

A multi-PUSCH configured grant has multiple consecutive configured uplink grants within a periodicity. Both Type 1 and Type 2 can be configured for a multi-PUSCH configured grant by RRC.

…

RRC configures the following parameters when the configured grant Type 2 is configured:

-	cs-RNTI: CS-RNTI for activation, deactivation, and retransmission;

-	periodicity: periodicity of the configured grant Type 2;

-	nrofHARQ-Processes: the number of HARQ processes for configured grant;

-	harq-ProcID-Offset: offset of HARQ process for configured grant configured with cg-RetransmissionTimer for operation with shared spectrum channel access;

-	harq-ProcID-Offset2: offset of HARQ process for configured grant not configured with cg-RetransmissionTimer.

…

RRC configures the following parameter when a multi-PUSCH configured grant is configured:

-	nrofSlotsInCG-Period: the number of configured uplink grants in a periodicity of a multi-PUSCH configured grant.

….

After an uplink grant is configured for a configured grant Type 2, the MAC entity shall consider sequentially that the configured uplink grant, or the first configured uplink grant in a multi-PUSCH configured grant, in the Nth (N ≥ 0) periodicity occurs in the symbol for which:

[(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)
	+ (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] =
	[(SFNstart time × numberOfSlotsPerFrame × numberOfSymbolsPerSlot
	+ slotstart time × numberOfSymbolsPerSlot + symbolstart time) + N × periodicity]
	modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)

where SFNstart time, slotstart time, and symbolstart time are the SFN, slot, and symbol, respectively, of the first transmission opportunity of PUSCH where the configured uplink grant was (re-)initialised.

For a multi-PUSCH configured grant Type 2, the Mth (1 < M ≤ nrofSlotsInCG-Period) configured uplink grant within the same periodicity occurs (M-1) × numberOfSymbolsPerSlot symbols after the symbol in which the first configured uplink grant in that periodicity occurs.

…

When the configured uplink grant is released by upper layers, all the corresponding configurations shall be released and all corresponding uplink grants shall be cleared.

The MAC entity shall:

1>	if at least one configured uplink grant confirmation has been triggered and not cancelled; and

1>	if the MAC entity has UL resources allocated for new transmission:

2>	if, in this MAC entity, at least one configured uplink grant is configured by configuredGrantConfigToAddModList:

3>	instruct the Multiplexing and Assembly procedure to generate a Multiple Entry Configured Grant Confirmation MAC CE as defined in clause 6.1.3.31.

2>	else:

3>	instruct the Multiplexing and Assembly procedure to generate a Configured Grant Confirmation MAC CE as defined in clause 6.1.3.7.

2>	cancel all triggered configured uplink grant confirmation(s).

For a configured grant Type 2, the MAC entity shall clear the configured uplink grant(s) immediately after first transmission of Configured Grant Confirmation MAC CE or Multiple Entry Configured Grant Confirmation MAC CE which confirms the configured uplink grant deactivation.

Retransmissions use:

-	repetition of configured uplink grants; or

-	received uplink grants addressed to CS-RNTI; or

-	configured uplink grants with cg-RetransmissionTimer, cg-RRC-RetransmissionTimer or cg-SDT-RetransmissionTimer configured.

[TS 38.321, clause 16.15.4.1]

The following enhancements for configured grant-based PUSCH transmission are introduced:

-	Support of multiple CG PUSCH transmission occasions within a single period of a CG configuration;

-	Indication of unused CG PUSCH occasion(s) of a CG configuration with Uplink Control Information multiplexed in CG PUSCH transmission of the CG configuration.

[TS 38.213, clause 10.2]

If a UE is provided a single configuration for UL grant Type 2 PUSCH or for SPS PDSCH, validation of the DCI format is achieved if all fields for the DCI format are set according to Table 10.2-1 or Table 10.2-2.

Table 10.2-1: Special fields for single DL SPS or single UL grant Type 2 scheduling activation PDCCH validation when a UE is provided a single SPS PDSCH or UL grant Type 2 configuration in the active DL/UL BWP of the scheduled cell

|  | DCI format 0_0/0_2 | DCI format 0_1 | DCI format 1_0/1_2/4_1 | DCI format 1_1/4_2 |
| --- | --- | --- | --- | --- |
| HARQ process number (if present) | set to all '0's | set to all '0's | set to all '0's | set to all '0's |
| Redundancy version (if present) | set to all '0's | For the enabled transport block: set to all '0's | set to all '0's | For the enabled transport block: set to all '0's |

7.1.1.6.7.3	Test description

7.1.1.6.7.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 and UM DRB should be established on NR Cell 1.The loop back size is set to accommodate one RLC SDU in UL of same size as one RLC SDU in DL and 1 byte MAC subheader for Configured Grant Confirmation MAC CE.

7.1.1.6.7.3.2	Test procedure sequence

Table 7.1.1.6.7.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits NR RRCReconfiguration message to configure UL configured grant type 2. | <-- | (NR RRC: RRCReconfiguration) | - | - |
| 2 | The UE transmits NR RRCReconfigurationComplete message. | --> | (NR RRC: RRCReconfigurationComplete) | - | - |
| 3 | SS transmits a DL MAC PDU containing 4 RLC SDUs (RLC SDU #1, RLC SDU #2, RLC SDU #3 and RLC SDU #4) on UM DRB. The size of each RLC SDU is 99 bytes | <-- | MAC PDU (four RLC SDUs) | - | - |
| 4 | The UE transmits a Scheduling Request, indicating that loop back SDUs are ready for transmission in UL RLC. | --> | (SR) | - | - |
| 5 | SS transmits a UL configured grant type 2 addressed to UE’s stored CS-RNTI in Slot ‘n’, SFN ‘m’ of PDCCH, NDI=0, allowing the UE to transmit one loop back SDU and 1 byte MAC subheader for Configured Grant Confirmation MAC CE. (Note 1) | <-- | (UL configured Grant type 2) | - | - |
| 6 | Check: Does the UE transmit a MAC PDU containing RLC SDU #1 received in step 3 and a Configured Grant Confirmation MAC CE in Slot ’n+K2’, SFN ‘m’? (Note 2) | --> | MAC PDU (RLC SDU #1 and Configured Grant Confirmation MAC CE) | 1 | P |
| 7 | Check: Does the UE transmit a MAC PDU containing RLC SDU #2 received in step 3 in the next UL slot after slot ‘n+K2’, SFN ‘m’? (Note 2) | --> | MAC PDU (RLC SDU #2) | 3 | P |
| 8 | Check: Does the UE transmit a MAC PDU containing RLC SDU #3 received in step 3 in slot ‘n+K2’, SFN ‘m+4’? (Note 2) | --> | MAC PDU (RLC SDU #3) | 2 | P |
| 9 | Check: Does the UE transmit a MAC PDU containing RLC SDU #4 received in step 3 in the next UL slot after slot ‘n+K2’, SFN ‘m+4’? (Note 2) | --> | MAC PDU (RLC SDU #4) | 3 | P |
| 10 | SS transmits a UL grant addressed to UE’s stored CS-RNTI with NDI set as 1 and HARQ process id set to be the same as step 9 in SFN ‘m+4’ , allowing the UE to retransmit one loop back SDU.(Note 3) | <-- | (UL Grant) | - | - |
| 11 | Check: Does the UE retransmit the MAC PDU containing RLC SDU #4 as in step 9. | --> | MAC PDU (RLC SDU #4) | 4 | P |
| 12 | The SS transmits NR RRCReconfiguration message to release UL configured grant type 2. | <-- | (NR RRC: RRCReconfiguration | - | - |
| 13 | The UE transmits NR RRCReconfigurationComplete message. | --> | (NR RRC: RRCReconfigurationComplete | - | - |
| Note 1: TB size for configured grant type 2 is 848 bits, which is enough to allow the UE to transmit one PDU at a time (99 bytes RLC SDU + 1 byte UM RLC Header + 2 bytes MAC Sub PDU header + 2 bytes for short BSR or padding + 1 byte MAC subheader for Configured Grant Confirmation MAC CE). Note 2: For (TDD AND SCS15) OR SCS120, n=0 and K2=4, the valid CG PUSCH occasion is Slot 4 and Slot 9; For other condition, n=4 and K2=4, the valid CG PUSCH occasion is Slot 8 and Slot 9. Note 3: HARQ process id is calculated based on the formula “HARQ Process ID = [nrofSlotsInCG-Period× floor (CURRENT_symbol / periodicity) + ID_OFFSET] modulo nrofHARQ-Processes” specified in TS 38.321 5.4.1. |  |  |  |  |  |

7.1.1.6.7.3.3	Specific message contents

Table 7.1.1.6.7.3.3-1: RRCReconfiguration (step 1, Table 7.1.1.6.7.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | Not present |  |  |
| secondaryCellGroup | Not Present |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| dedicatedNAS-MessageList | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.6.7.3.3-2: CellGroupConfig (Table 7.1.1.6.7.3.3-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList | Not present |  |  |
| mac-CellGroupConfig | Not present |  |  |
| physicalCellGroupConfig SEQUENCE { |  |  |  |
| cs-RNTI CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| RNTI-Value | ‘FFE0’H |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync | Not present |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP SEQUENCE { |  |  |  |
| configuredGrantConfig | ConfiguredGrantConfig | Table 7.1.1.6.7.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.6.7.3.3-3: ConfiguredGrantConfig (Table 7.1.1.6.7.3.3-2)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConfiguredGrantConfig ::= SEQUENCE { |  |  |  |
| uci-OnPUSCH CHOICE { |  |  |  |
| setup CHOICE { |  |  |  |
| semiStatic | BetaOffsets | TS 38.508-1[4], Table 4.6.3-6A |  |
| } |  |  |  |
| } |  |  |  |
| cg-RetransmissionTimer-r16 | Not present |  |  |
| cg-minDFI-Delay-r16 | Not present |  |  |
| cg-nrofPUSCH-InSlot-r16 | Not present |  |  |
| cg-nrofSlots-r16 | Not present |  |  |
| cg-UCI-Multiplexing-r16 | Not present |  |  |
| cg-COT-SharingOffset-r16 | Not present |  |  |
| betaOffsetCG-UCI-r16 | Not present |  |  |
| cg-COT-SharingList-r16 | Not present |  |  |
| harq-ProcID-Offset-r16 | Not present |  |  |
| harq-ProcID-Offset2-r16 | Not present |  |  |
| configuredGrantConfigIndex-r16 | Not present |  |  |
| configuredGrantConfigIndexMAC-r16 | Not present |  |  |
| periodicityExt-r16 | Not present |  |  |
| startingFromRV0-r16 | Not present |  |  |
| phy-PriorityIndex-r16 | Not present |  |  |
| autonomousTx-r16 | Not present |  |  |
| cg-betaOffsetsCrossPri0-r17 | Not present |  |  |
| cg-betaOffsetsCrossPri1-r17 | Not present |  |  |
| mappingPattern-r17 | Not present |  |  |
| sequenceOffsetForRV-r17 | Not present |  |  |
| p0-PUSCH-Alpha2-r17 | Not present |  |  |
| powerControlLoopToUse2-r17 | Not present |  |  |
| cg-COT-SharingList-r17 | Not present |  |  |
| periodicityExt-r17 | Not present |  |  |
| repK-v1710 | Not present |  |  |
| nrofHARQ-Processes-v1700 | Not present |  |  |
| harq-ProcID-Offset2-v1700 | Not present |  |  |
| configuredGrantTimer-v1700 | Not present |  |  |
| cg-minDFI-Delay-v1710 | Not present |  |  |
| harq-ProcID-Offset-v1730 | Not present |  |  |
| cg-nrofSlots-r17 | Not present |  |  |
| disableCG-RetransmissionMonitoring-r18 | Not present |  |  |
| nrofSlotsInCG-Period-r18 | 6 |  | (TDD AND SCS15) OR SCS120 |
|  | 2 |  |  |
| uto-UCI-Config-r18 | Not present |  |  |
| } |  |  |  |

Table 7.1.1.6.7.3.3-4: RRCReconfiguration (step 12, Table 7.1.1.6.7.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | Not present |  |  |
| secondaryCellGroup | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| dedicatedNAS-MessageList | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.6.7.3.3-5: CellGroupConfig (Table 7.1.1.6.7.3.3-4)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP SEQUENCE { |  |  |  |
| configuredGrantConfig CHOICE { |  |  |  |
| release | Null |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 7.1.1.6.8	Correct handling of UL grant / Unused transmission occasion / Configured grant type 1

7.1.1.6.8.1	Test Purpose (TP)

(1)

with { UE in RRC_Connected state with DRB established and configured UL grant type 1 }

ensure that {

when { the UE determines that buffered data can be transmitted using N CG occasion (N>0, N<nrofBitsInUTO-UCI-r18) and UE transmits for the first time in CG occasion m }

then { UE transmits UTO-UCI that indicates the usage status (used/unused) of each transmission occasion. The first N-X-1 bits of UTO-UCI are set to 0 (used) and bits from N-X to nrofBitsInUTO-UCI-r18 set to either 1 (unused) or 0 (used) }

}

7.1.1.6.8.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 38.321 clause 5.8.2. Unless otherwise stated these are Rel-18 requirements.

[TS 38.321, clause 5.8.2]

RRC configures the following parameter when UTO-UCI (as specified in clause 9.3 in TS 38.213 [6]) is configured for a configured grant:

-	nrofBitsInUTO-UCI: number of bits in a UTO-UCI bitmap.

For a configured uplink grant, the MAC entity shall:

1>	if its associated configured grant is configured with UTO-UCI and it has not been indicated to the lower layers as unused for PUSCH transmission; or

1>	if its associated configured grant is not configured with UTO-UCI:

2>	if it is associated with a multi-PUSCH configured grant and meets the validity conditions specified in the clause 6.1 in TS 38.214 [7]; or

2>	if it is not associated with a multi-PUSCH configured grant:

3>	consider it available for use.

The MAC entity shall not include the UL-SCH resource of a configured uplink grant not available for use in its procedures (e.g. in clause 5.4.4).

For a configured grant configured with UTO-UCI, the MAC entity determines if a configured uplink grant which is within the subsequent nrofBitsInUTO-UCI valid occasions of its associated configured grant configuration is going to be used for PUSCH transmission by considering at least the amount of buffered data that can be transmitted on the available occasions of the associated configured grant and other available UL-SCH resources. Upon this determination, the MAC entity sends an indication to lower layers, for use in the procedure for reporting UTO-UCI.

7.1.1.6.8.3	Test description

7.1.1.6.8.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0.

7.1.1.6.8.3.2	Test procedure sequence

Table 7.1.1.6.8.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits RRCReconfiguration message to configure UL configured grant type 1 and UTO-UCI for 8 transmission occasions. | <-- | RRCReconfiguration | - | - |
| 2 | The UE transmits RRCReconfigurationComplete message to indicate successful configuration. | --> | RRCReconfigurationComplete | - | - |
| 3 | The SS transmits a DL MAC PDU containing 4 RLC SDUs of size 99 bytes. (Note 1) | <-- | MAC PDU (4 RLC SDUs) | - | - |
| - | EXCEPTION: Step 4 is repeated 4 times. Testcase is considered passed when UE transmits UTO-UCI with first 4-m-1 bits set to 0 and last 5+m bits set to either 0 or 1, where m is the index of transmission occasion, i.e. m = [0,1,2,3]. | - | - | - | - |
| 4 | Check: Does the UE transmit a UTO-UCI that indicates the usage status (used/unused) of each transmission occasion, along with a UL MAC PDU, on the scheduled transmission occasion? | --> | MAC PDU (RLC SDU) UTO-UCI | 1 | P |
| Note 1: According to the setting parameters in Table 7.1.1.6.8.3.3-2, TB size for configured grant type 1 is 848 bits, which is enough to allow the UE to transmit one PDU at a time (99 bytes RLC SDU + 1 byte UM RLC Header + 2 bytes MAC Sub PDU header + 2 bytes for short BSR or padding). |  |  |  |  |  |

7.1.1.6.8.3.3	Specific message contents

Table 7.1.1.6.8.3.3-1: RRCReconfiguration (step 1, Table 7.1.1.6.8.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.6.8.3.3-2: CellGroupConfig (Table 7.1.1.6.8.3.3-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList | Not present |  |  |
| mac-CellGroupConfig | Not present |  |  |
| physicalCellGroupConfig SEQUENCE { |  |  |  |
| cs-RNTI CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| RNTI-Value | ‘FFE0’H |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| servCellIndex | Not present |  |  |
| reconfigurationWithSync | Not present |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP SEQUENCE { |  |  |  |
| pucch-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| schedulingRequestResourceToAddModList SEQUENCE (SIZE (1..maxNrofSR-Resources)) OF SchedulingRequestResourceConfig { | 1 entry |  |  |
| SchedulingRequestResourceConfig[1] SEQUENCE { |  | entry 1 |  |
| schedulingRequestResourceId | 1 |  |  |
| schedulingRequestID | 0 |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| sl20 | 9 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| configuredGrantConfig CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cg-DMRS-Configuration | DMRS-UplinkConfig | Reference TS 38.508-1[4], Table 4.6.3-51 |  |
| uci-OnPUSCH CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| semiStatic SEQUENCE { | BetaOffsets |  |  |
| betaOffsetACK-Index1 | 9 |  |  |
| betaOffsetACK-Index2 | 9 |  |  |
| betaOffsetACK-Index3 | 9 |  |  |
| betaOffsetCSI-Part1-Index1 | 6 |  |  |
| betaOffsetCSI-Part1-Index2 | 6 |  |  |
| betaOffsetCSI-Part2-Index1 | 6 |  |  |
| betaOffsetCSI-Part2-Index2 | 6 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| resourceAllocation | ResourceAllocationType1 |  |  |
| powerControlLoopToUse | n0 |  |  |
| p0-PUSCH-Alpha | 1 |  |  |
| nrofHARQ-Processes | 16 |  |  |
| repK | n1 |  |  |
| periodicity | Sym40x14 |  | 15kHz |
| periodicity | Sym80x14 |  | 30kHz |
| periodicity | Sym160x14 |  | 60kHz |
| periodicity | Sym320x14 |  | 120kHz |
| rrc-ConfiguredUplinkGrant SEQUENCE { |  |  |  |
| timeDomainOffset | 9 |  |  |
| timeDomainAllocation | 0 | Reference TS 38.508-1 [4], Table 4.6.3-122 |  |
| frequencyDomainAllocation | BIT STRING (SIZE(18) | BIT STRING (SIZE(18), Equal to NBWPsize * (LRB-1) + RBstart), where LRB = 23 PRB, RBstart = 0, NBWPsize is the size [PRBs] of the active carrier bandwidth part and ontained in TS.38.508-1 [4] clause 4.3.1.1. | FR1_FDD, FR1_TDD |
|  | BIT STRING (SIZE(18) | BIT STRING (SIZE(18), Equal to NBWPsize * (LRB-1) + RBstart), where LRB = 14 PRB, RBstart = 0, NBWPsize is the size [PRBs] of the active carrier bandwidth part and ontained in TS.38.508-1 [4] clause 4.3.1.1. | FR1_FDD AND BW3 |
|  | BIT STRING (SIZE(18) | BIT STRING (SIZE(18), Equal to NBWPsize * (LRB-1) + RBstart), where LRB=23 PRB, RBstart = 0and NBWPsize is the size [PRBs] of the active carrier bandwidth part and ontained in TS.38.508-1 [4] clause 4.3.1.2. | FR2_TDD |
| antennaPort | 0 |  |  |
| precodingAndNumberOfLayers | 0 |  |  |
| srs-ResourceIndicator | Not present |  |  |
| mcsAndTBS | 0 |  | FR1_FDD, FR1_TDD |
|  | 0 |  | FR2_TDD |
|  | 2 |  | FR1_FDD AND BW3 |
| pathlossReferenceIndex | 0 |  |  |
| } |  |  |  |
| uto-UCI-Config-r18 SEQUENCE { |  |  |  |
| nrofBitsInUTO-UCI-r18 | 8 |  |  |
| betaOffsetUTO-UCI-r18 | 9 |  |  |
| } |  |  |  |
| } |  |  |  |
| pusch-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| pusch-TimeDomainAllocationList CHOICE { |  |  |  |
| setup SEQUENCE { | 1 entry |  |  |
| PUSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  | entry 1 |  |
| k2 | n4 |  | FR1 |
|  | n8 |  | FR2 |
| mappingType | typeB |  |  |
| startSymbolAndLength | 0011011 | Start symbol(S)=0, Length(L)=14 | FR1 |
| startSymbolAndLength | 0001110 | S=0, L=2 | FR2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 7.1.1.6.9	Correct handling of UL grant / Unused transmission occasion / Configured grant type 2

7.1.1.6.9.1	Test Purpose (TP)

(1)

with { UE in RRC_Connected state with DRB established and configured UL grant type 2 }

ensure that {

when { the UE determines that buffered data can be transmitted using N CG occasion (N>0, N<nrofBitsInUTO-UCI-r18) and UE transmits for the first time in CG occasion m }

then { UE transmits UTO-UCI that indicates the usage status (used/unused) of each transmission occasion. The first N-X-1 bits of UTO-UCI are set to 0 (used) and bits from N-X to nrofBitsInUTO-UCI-r18 set to either 1 (unused) or 0 (used) }

}

7.1.1.6.9.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 38.321 clause 5.8.2. Unless otherwise stated these are Rel-18 requirements.

[TS 38.321, clause 5.8.2]

RRC configures the following parameter when UTO-UCI (as specified in clause 9.3 in TS 38.213 [6]) is configured for a configured grant:

-	nrofBitsInUTO-UCI: number of bits in a UTO-UCI bitmap.

For a configured uplink grant, the MAC entity shall:

1>	if its associated configured grant is configured with UTO-UCI and it has not been indicated to the lower layers as unused for PUSCH transmission; or

1>	if its associated configured grant is not configured with UTO-UCI:

2>	if it is associated with a multi-PUSCH configured grant and meets the validity conditions specified in the clause 6.1 in TS 38.214 [7]; or

2>	if it is not associated with a multi-PUSCH configured grant:

3>	consider it available for use.

The MAC entity shall not include the UL-SCH resource of a configured uplink grant not available for use in its procedures (e.g. in clause 5.4.4).

For a configured grant configured with UTO-UCI, the MAC entity determines if a configured uplink grant which is within the subsequent nrofBitsInUTO-UCI valid occasions of its associated configured grant configuration is going to be used for PUSCH transmission by considering at least the amount of buffered data that can be transmitted on the available occasions of the associated configured grant and other available UL-SCH resources. Upon this determination, the MAC entity sends an indication to lower layers, for use in the procedure for reporting UTO-UCI.

7.1.1.6.9.3	Test description

7.1.1.6.9.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0.

7.1.1.6.9.3.2	Test procedure sequence

Table 7.1.1.6.9.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message to configure UL configured grant type 2 and UTO-UCI for 8 transmission occasions. | <-- | RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message to indicate successful configuration. | --> | RRCReconfigurationComplete | - | - |
| 3 | The SS transmits a DL MAC PDU containing 4 RLC SDUs of size 99 bytes. (Note 1) | <-- | MAC PDU (4 RLC SDUs) | - | - |
| - | EXCEPTION: UE may send Scheduling Request as a mark of indicating that loop back SDUs are ready for transmission in UL RLC. | - | - | - | - |
| 4 | SS transmits a UL configured grant type 2 addressed to UE’s stored CS-RNTI, allowing the UE to transmit one loop back SDU and 1 byte MAC subheader for Configured Grant Confirmation MAC CE. | <-- | (UL configured Grant type 2) | - | - |
| - | EXCEPTION: Step 5 is repeated 4 times. Testcase is considered passed when UE transmits UTO-UCI with first 4-m-1 bits set to 0 and last 5+m bits set to either 0 or 1, where m is the index of transmission occasion, i.e. m = [0,1,2,3]. | - | - | - | - |
| 5 | Check: Does the UE transmit a UTO-UCI that indicates the usage status (used/unused) of each transmission occasion, along with a UL MAC PDU, on the scheduled transmission occasion? | --> | MAC PDU (RLC SDU) UTO-UCI | 1 | P |
| Note 1: According to the setting parameters in Table 7.1.1.6.9.3.3-2, TB size for configured grant type 2 is 848 bits, which is enough to allow the UE to transmit one PDU at a time (99 bytes RLC SDU + 1 byte UM RLC Header + 2 bytes MAC Sub PDU header + 2 bytes for short BSR or padding). |  |  |  |  |  |

7.1.1.6.9.3.3	Specific message contents

Table 7.1.1.6.9.3.3-1: RRCReconfiguration (step 1, Table 7.1.1.6.9.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.6.9.3.3-2: CellGroupConfig (Table 7.1.1.6.9.3.3-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList | Not present |  |  |
| mac-CellGroupConfig | Not present |  |  |
| physicalCellGroupConfig SEQUENCE { |  |  |  |
| cs-RNTI CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| RNTI-Value | ‘FFE0’H |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| servCellIndex | Not present |  |  |
| reconfigurationWithSync | Not present |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP SEQUENCE { |  |  |  |
| pucch-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| schedulingRequestResourceToAddModList SEQUENCE (SIZE (1..maxNrofSR-Resources)) OF SchedulingRequestResourceConfig { | 1 entry |  |  |
| SchedulingRequestResourceConfig[1] SEQUENCE { |  | entry 1 |  |
| schedulingRequestResourceId | 1 |  |  |
| schedulingRequestID | 0 |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| sl20 | 9 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| configuredGrantConfig CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cg-DMRS-Configuration | DMRS-UplinkConfig | Reference TS 38.508-1[4], Table 4.6.3-51 |  |
| uci-OnPUSCH CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| semiStatic SEQUENCE { | BetaOffsets |  |  |
| betaOffsetACK-Index1 | 9 |  |  |
| betaOffsetACK-Index2 | 9 |  |  |
| betaOffsetACK-Index3 | 9 |  |  |
| betaOffsetCSI-Part1-Index1 | 6 |  |  |
| betaOffsetCSI-Part1-Index2 | 6 |  |  |
| betaOffsetCSI-Part2-Index1 | 6 |  |  |
| betaOffsetCSI-Part2-Index2 | 6 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| resourceAllocation | ResourceAllocationType1 |  |  |
| powerControlLoopToUse | n0 |  |  |
| p0-PUSCH-Alpha | 0 |  |  |
| nrofHARQ-Processes | 16 |  |  |
| repK | n1 |  |  |
| periodicity | Sym40x14 |  | 15kHz |
| periodicity | Sym80x14 |  | 30kHz |
| periodicity | Sym160x14 |  | 60kHz |
| periodicity | Sym320x14 |  | 120kHz |
| uto-UCI-Config-r18 SEQUENCE { |  |  |  |
| nrofBitsInUTO-UCI-r18 | 8 |  |  |
| betaOffsetUTO-UCI-r18 | 9 |  |  |
| } |  |  |  |
| } |  |  |  |
| pusch-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| pusch-TimeDomainAllocationList CHOICE { |  |  |  |
| setup SEQUENCE { | 1 entry |  |  |
| PUSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  | entry 1 |  |
| k2 | n4 |  | FR1 |
|  | n8 |  | FR2 |
| mappingType | typeB |  |  |
| startSymbolAndLength | 0011011 | Start symbol(S)=0, Length(L)=14 | FR1 |
| startSymbolAndLength | 0001110 | S=0, L=2 | FR2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 7.1.1.7	Activation/Deactivation of SCells

##### 7.1.1.7.1	Activation/Deactivation of SCells / Activation/Deactivation MAC control element reception / sCellDeactivationTimer

###### 7.1.1.7.1.1	Activation/Deactivation of SCells / Activation/Deactivation MAC control element reception / sCellDeactivationTimer / Intra-band Contiguous CA

7.1.1.7.1.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with SCell configured }

ensure that {

when { UE receives an SCell Activation/Deactivation MAC CE activating the Scell }

then { UE starts monitoring PDCCH on activated SCell }

}

(2)

with( UE in RRC_CONNECTED state with SCell activated )

ensure that {

when{ UE receives a DL assignment on SCell PDCCH }

then {  UE restarts the sCellDeactivationTimer }

}

(3)

with ( UE in RRC_CONNECTED state with SCell activated )

ensure that {

when{ UE sCellDeactivationTimer expires }

then { UE deactivates the SCell and stops monitoring PDCCH on SCell }

}

(4)

with ( UE in RRC_CONNECTED state with SCell activated )

ensure that {

when{ UE receives a SCell Activation/Deactivation MAC CE deactivating the SCell }

then { UE deactivates the SCell and stops monitoring PDCCH on SCell }

}

(5)

with( UE in RRC_CONNECTED state with SCell activated and UL CA is supported )

ensure that {

when{ UE receives a UL assignment on SCell and has data available for transmission }

then { UE transmits the UL MAC PDU }

}

7.1.1.7.1.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.321, clauses 5.9 and TS 38.331 clause 5.3.5.5.2. Unless otherwise stated these are Rel-15 requirements.

[TS 38.321, clause 5.9]

If the MAC entity is configured with one or more SCells, the network may activate and deactivate the configured SCells. Upon configuration of an SCell, the SCell is deactivated.

The configured SCell(s) is activated and deactivated by:

-	receiving the SCell Activation/Deactivation MAC CE described in subclause 6.1.3.10;

-	configuring sCellDeactivationTimer timer per configured SCell (except the SCell configured with PUCCH, if any): the associated SCell is deactivated upon its expiry.

The MAC entity shall for each configured SCell:

1>	if an SCell Activation/Deactivation MAC CE is received activating the SCell:

2>	activate the SCell according to the timing defined in TS 38.213 [6]; i.e. apply normal SCell operation including:

3>	SRS transmissions on the SCell;

3>	CSI reporting for the SCell;

3>	PDCCH monitoring on the SCell;

3>	PDCCH monitoring for the SCell;

3>	PUCCH transmissions on the SCell, if configured.

2>	start or restart the sCellDeactivationTimer associated with the SCell in the slot when the SCell Activation/Deactivation MAC CE was received;

2>	(re-)initialize any suspended configured uplink grants of configured grant Type 1 associated with this SCell according to the stored configuration, if any, and to start in the symbol according to rules in subclause 5.8.2;

2>	trigger PHR according to subclause 5.4.6.

1>	else if an SCell Activation/Deactivation MAC CE is received deactivating the SCell; or

1>	if the sCellDeactivationTimer associated with the activated SCell expires:

2>	deactivate the SCell according to the timing defined in TS 38.213 [6];

2>	stop the sCellDeactivationTimer associated with the SCell;

2>	stop the bwp-InactivityTimer associated with the SCell;

2>	clear any configured downlink assignment and any configured uplink grant Type 2 associated with the SCell respectively;

2>	suspend any configured uplink grant Type 1 associated with the SCell;

2>	flush all HARQ buffers associated with the SCell.

1>	if PDCCH on the activated SCell indicates an uplink grant or downlink assignment; or

1>	if PDCCH on the Serving Cell scheduling the activated SCell indicates an uplink grant or a downlink assignment for the activated SCell; or

1>	if a MAC PDU is transmitted in a configured uplink grant or received in a configured downlink assignment:

2>	restart the sCellDeactivationTimer associated with the SCell.

1>	if the SCell is deactivated:

2>	not transmit SRS on the SCell;

2>	not report CSI for the SCell;

2>	not transmit on UL-SCH on the SCell;

2>	not transmit on RACH on the SCell;

2>	not monitor the PDCCH on the SCell;

2>	not monitor the PDCCH for the SCell;

2>	not transmit PUCCH on the SCell.

HARQ feedback for the MAC PDU containing SCell Activation/Deactivation MAC CE shall not be impacted by PCell, PSCell and PUCCH SCell interruptions due to SCell activation/deactivation in TS 38.133 [11].

When SCell is deactivated, the ongoing Random Access procedure on the SCell, if any, is aborted.

[TS 38.321, clause 6.1.3.10]

The SCell Activation/Deactivation MAC CE of one octet is identified by a MAC PDU subheader with LCID as specified in Table 6.2.1-1. It has a fixed size and consists of a single octet containing seven C-fields and one R-field. The SCell Activation/Deactivation MAC CE with one octet is defined as follows (Figure 6.1.3.10-1).

The SCell Activation/Deactivation MAC CE of four octets is identified by a MAC PDU subheader with LCID as specified in Table 6.2.1-1. It has a fixed size and consists of four octets containing 31 C-fields and one R-field. The SCell Activation/Deactivation MAC CE of four octets is defined as follows (Figure 6.1.3.10-2).

For the case with no Serving Cell with a ServCellIndex as specified in TS 38.331 [8] larger than 7, SCell Activation/Deactivation MAC CE of one octet is applied, otherwise SCell Activation/Deactivation MAC CE of four octets is applied.

-	Ci: If there is an SCell configured for the MAC entity with SCellIndex i as specified in TS 38.331 [8], this field indicates the activation/deactivation status of the SCell with SCellIndex i, else the MAC entity shall ignore the Ci field. The Ci field is set to "1" to indicate that the SCell with SCellIndex i shall be activated. The Ci field is set to "0" to indicate that the SCell with SCellIndex i shall be deactivated;

-	R: Reserved bit, set to "0".

Figure 6.1.3.10-1: SCell Activation/Deactivation MAC CE of one octet

Figure 6.1.3.10-2: SCell Activation/Deactivation MAC CE of four octets

7.1.1.7.1.1.3	Test description

7.1.1.7.1.1.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that DRB is configured in RLC AM mode according to Table 7.1.1. 7.1.1.3.1-1 and in additionNR Cell 3 is configured as NR Active SCell.

Table 7.1.1. 7.1.1.3.1-1: RLC parameters

| t-PollRetransmit | ms80 |
| --- | --- |

7.1.1.7.1.1.3.2	Test procedure sequence

Table 7.1.1.7.1.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 |
| --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -85 | -85 |

Table 7.1.1.7.1.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits an RRCReconfiguration message to configure NR SCell(NR Cell 3). Note 1 | <-- | (RRCReconfiguration) | - | - |
| 2 | The UE transmits RRCReconfigurationComplete message. Note 2 | --> | (RRCReconfigurationComplete) | - | - |
| 3 | The SS transmits Activation MAC control element to activate NR SCell on NR SpCell. | <-- | MAC PDU (SCell Activation/Deactivation MAC CE of one octet (C1=1)) | - | - |
| 4 | 200 ms after step 3, the SS indicates a new transmission on PDCCH of SCell and transmits a MAC PDU (containing an RLC PDU). | <-- | MAC PDU | - | - |
| 5 | Check: Does the UE transmit a Scheduling Request on PUCCH? | --> | (SR) | 1 | P |
| 6 | The SS sends an UL grant suitable for transmitting loop back PDU on NR SpCell. | <-- | (UL Grant) | - | - |
| 7 | The UE transmits a MAC PDU containing the loop back PDU corresponding to step 4. | --> | MAC PDU | - | - |
| 8 | The SS transmits a MAC PDU containing RLC status PDU acknowledging reception of RLC PDU in step 7 on NR SpCell. | <-- | MAC PDU | - | - |
| 9 | 400 ms(sCellDeactivationTimer = 320 ms) after step 4, the SS indicates a new transmission on PDCCH of NR SCell and transmits a MAC PDU (containing an RLC PDU). | <-- | MAC PDU | - | - |
| 10 | Check: Does the UE transmit a Scheduling Request on PUCCH in next 1 second? | --> | (SR) | 3 | F |
| 11 | The SS transmits Activation MAC control element to activate SCell on NR SpCell. | <-- | MAC PDU ((SCell Activation/Deactivation MAC CE of one octet (C1=1)) | - | - |
| 12 | 200 ms after step 11 The SS indicates a new transmission on PDCCH of NR SCell and transmits a MAC PDU (containing just padding or RLC status PDU, but no RLC data PDU). | <-- | MAC PDU | - | - |
| 13 | 400 ms after step 11 the SS indicates a new transmission on PDCCH of NR SCell and transmits a MAC PDU (containing an RLC PDU). | <-- | MAC PDU | - | - |
| 14 | Check: Does the UE transmit a Scheduling Request on PUCCH on NR SpCell? | --> | (SR) | 1,2 | P |
| 15 | The SS sends an UL grant suitable for transmitting loop back PDU IF (pc_UL_NR_CA_2CC for NR or pc_EN_DC_NR_UL_2CC for EN-DC) on NR SCell ELSE the SS sends an UL grant suitable for transmitting loop back PDU on NR SpCell. | <-- | (UL Grant) | - | - |
| 16 | The UE transmits a MAC PDU containing the loop back PDU corresponding to step 13 IF (pc_UL_NR_CA_2CC for NR or pc_EN_DC_NR_UL_2CC for EN-DC) on NR SCell ELSE on NR SpCell. | --> | MAC PDU | 5 | P |
| 17 | The SS transmits a MAC PDU containing RLC status PDU acknowledging reception of RLC PDU in step 16 IF (pc_UL_NR_CA_2CC for NR or pc_EN_DC_NR_UL_2CC for EN-DC) on NR SCell ELSE on NR SpCell. | <-- | MAC PDU | - | - |
| 18 | The SS transmits Deactivation MAC control element to de-activate SCell on NR SpCell. | <-- | MAC PDU (SCell Activation/Deactivation MAC CE of one octet (C1=0)) | - | - |
| 19 | The SS indicates a new transmission on PDCCH of NR SCell and transmits a MAC PDU (containing an RLC PDU). | <-- | MAC PDU | - | - |
| 20 | Check: Does the UE transmit a Scheduling Request on PUCCH in the next 1 second? | --> | (SR) | 4 | F |
| Note 1: For EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration. Note 2: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. |  |  |  |  |  |

7.1.1.7.1.1.3.3	Specific message contents

Table 7.1.1.7.1.1.3.3-1: RRCReconfiguration (step 1, Table 7.1.1.7.1.1.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| secondaryCellGroup |  | CellGroupConfig |  | EN-DC |
| nonCriticalExtension SEQUENCE { |  |  |  | NR |
| masterCellGroup |  | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.7.1.1.3.3-2: CellGroupConfig (Table 7.1.1.7.1.1.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SCellConfig { |  | 1 entry |  |  |
| SCellConfig[1] SEQUENCE { |  |  | entry 1 |  |
| sCellIndex |  | SCellIndex as per TS 38.508-1 [4] table 4.6.3-154 |  |  |
| sCellConfigCommon |  | ServingCellConfigCommon |  |  |
| sCellConfigDedicated |  | ServingCellConfig |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.7.1.1.3.3-3: ServingCellConfigCommon (Table 7.1.1.7.1.1.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| physCellId | Physical Cell Identity of NR Cell 3 |  |  |
| uplinkConfigCommon | Not present |  | Not pc_UL_NR_CA_2CC for NR or Not pc_EN_DC_NR_UL_2CC for EN-DC |
|  | UplinkConfigCommon as per TS 38.508-1 [4] table 4.6.3-201 |  | pc_UL_NR_CA_2CC for NR5GC or Not pc_EN_DC_NR_UL_2CC for EN-DC |
| } |  |  |  |

Table 7.1.1.7.1.1.3.3-4: ServingCellConfig (Table 7.1.1.7.1.1.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig | Not present |  | Not pc_UL_NR_CA_2CC for NR or Not pc_EN_DC_NR_UL_2CC for EN-DC |
| uplinkConfig SEQUENCE { |  |  | pc_UL_NR_CA_2CC for NR or Not pc_EN_DC_NR_UL_2CC for EN-DC |
| initialUplinkBWP | BWP-UplinkDedicated |  |  |
| } |  |  |  |
| sCellDeactivationTimer | ms320 |  |  |
| } |  |  |  |

Table 7.1.1.7.1.1.3.3-5: BWP-UplinkDedicated (Table 7.1.1.7.1.1.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicated ::= SEQUENCE { |  |  |  |
| pucch-Config | Not present |  |  |
| } |  |  |  |

Table 7.1.1.7.1.1.3.3-6: BWP-UplinkCommon (Table 7.1.1.7.1.1.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkCommon ::= SEQUENCE { |  |  |  |
| pucch-ConfigCommon | Not present |  |  |
| } |  |  |  |

###### 7.1.1.7.1.2	Activation/Deactivation of SCells / Activation/Deactivation MAC control element reception / sCellDeactivationTimer / Inter-Band CA

The scope and description of the present TC is the same as test case 7.1.1.7.1.1 with the following differences:

-	CA configuration: Inter-band CA replaces Intra-band Contiguous CA

-	Cells configuration: NR Cell 10 replaces NR Cell 3

###### 7.1.1.7.1.3	Activation/Deactivation of SCells / Activation/Deactivation MAC control element reception / sCellDeactivationTimer / Intra-band non-Contiguous CA

The scope and description of the present TC is the same as test case 7.1.1.7.1.1 with the following differences:

-	CA configuration: Intra-band non-Contiguous CA replaces Intra-band Contiguous CA

-	pc_UL_NR_CA_2CC or pc_EN_DC_NR_UL_2CC in specific message content is replaced by pc_UL_intra_non_contiguous_CA_NR_FR1_Class_(2A) for FR1 or pc_UL_intra_non_contiguous_CA_NR_FR2_Class_(2A) for FR2

##### 7.1.1.7.2	Activation/Deactivation of SCells / Multi-cell PDSCH scheduling with a single DCI / Search space for single DCI is only configured in scheduling cell / sCellDeactivationTimer

###### 7.1.1.7.2.1	Activation/Deactivation of SCells / Multi-cell PDSCH scheduling with a single DCI / Search space for single DCI is only configured in scheduling cell / sCellDeactivationTimer / Intra-band Contiguous CA

7.1.1.7.2.1.1	Test Purpose (TP)

(1)

with( UE in RRC_CONNECTED state with SCell activated, and the search space for single DCI is only configured in scheduling cell (i.e. the reference cell is scheduling cell) )

ensure that {

when{ UE receives a DCI format 1-3 on NR SpCell PDCCH to schedule SCell }

then {  UE restarts the sCellDeactivationTimer }

}

(2)

with ( UE in RRC_CONNECTED state with SCell activated, and the search space for single DCI is only configured in scheduling cell (i.e. the reference cell is scheduling cell))

ensure that {

when{ UE sCellDeactivationTimer expires }

then { UE does not stop monitoring PDCCH for scheduling multiple cells for the set of cells in MC-DCI-SetOfCells including the SCell (i.e. does not stop monitoring DCI format 1_3 on the scheduling cell for the set of cells) }

}

7.1.1.7.2.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.321, clauses 5.9 and TS 38.212, clauses 7.3.1.0. Unless otherwise stated these are Rel-18 requirements.

[TS 38.321, clause 5.9]

If the MAC entity is configured with one or more SCells, the network may activate and deactivate the configured SCells. Upon configuration of an SCell, the SCell is deactivated unless the parameter sCellState is set to activated for the SCell by upper layers.

The configured SCell(s) is activated and deactivated by:

-	receiving the SCell Activation/Deactivation MAC CE described in clause 6.1.3.10;

-	receiving the Enhanced SCell Activation/Deactivation MAC CE described in clause 6.1.3.55;

-	configuring sCellDeactivationTimer timer per configured SCell (except the SCell configured with PUCCH, if any): the associated SCell is deactivated upon its expiry;

-	configuring sCellState per configured SCell: if configured, the associated SCell is activated upon SCell configuration;

-	receiving scg-State: the SCells of SCG are deactivated.

The MAC entity shall for each configured SCell:

1>	if an SCell is configured with sCellState set to activated upon SCell configuration, or an SCell Activation/Deactivation MAC CE or an Enhanced SCell Activation/Deactivation MAC CE is received activating the SCell:

…

1>	else if an SCell Activation/Deactivation MAC CE or an Enhanced SCell Activation/Deactivation MAC CE is received deactivating the SCell; or

1>	if the sCellDeactivationTimer associated with the activated SCell expires; or

1>	if the SCG associated with the activated SCell is deactivated:

2>	deactivate the SCell according to the timing defined in TS 38.213 [6];

2>	stop the sCellDeactivationTimer associated with the SCell;

2>	stop the bwp-InactivityTimer associated with the SCell;

2>	deactivate any active BWP associated with the SCell;

2>	clear any configured downlink assignment and any configured uplink grant Type 2 associated with the SCell respectively;

2>	clear any PUSCH resource for semi-persistent CSI reporting associated with the SCell;

2>	suspend any configured uplink grant Type 1 associated with the SCell;

2>	flush all HARQ buffers associated with the SCell;

2>	cancel, if any, triggered consistent LBT failure for the SCell.

1>	if PDCCH on the activated SCell indicates an uplink grant or downlink assignment; or

1>	if PDCCH on the Serving Cell scheduling the activated SCell indicates an uplink grant or a downlink assignment for the activated SCell; or

1>	if a MAC PDU is transmitted in a configured uplink grant and LBT failure indication is not received from lower layers; or

1>	if a MAC PDU is received in a configured downlink assignment:

2>	restart the sCellDeactivationTimer associated with the SCell.

1>	if the SCell is deactivated:

2>	not transmit SRS on the SCell;

2>	not report CSI for the SCell;

2>	not transmit on UL-SCH on the SCell;

2>	not transmit on RACH on the SCell;

2>	not monitor the PDCCH on the SCell;

2>	not monitor the PDCCH for the SCell;

2>	not transmit PUCCH on the SCell;

2>	if the SCell is configured as a scheduled cell in MC-DCI-SetOfCells and with the search space for DCI to schedule multiple cells (as specified in TS 38.213 [6]) of the same searchSpaceId as the serving cell in which MC-DCI-SetOfCells containing the SCell is configured:

3>	not monitor the PDCCH for scheduling multiple cells (as specified in TS 38.213 [6]) for the set of cells in MC-DCI-SetOfCells including the SCell.

[TS 38.212, clause 7.3.1.0]

Step 3:

-	If both of the following conditions are fulfilled the size alignment procedure is complete

-	the total number of different DCI sizes configured to monitor is no more than 4 for the cell

-	the total number of different DCI sizes with C-RNTI configured to monitor is no more than 3 for the cell

Step 4:

-	Otherwise

Step 4A:

…

Step 4B:

…

Step 4C:

-	If the total number of different DCI sizes configured to monitor is more than 4 for the cell after applying the above steps, or if the total number of different DCI sizes with C-RNTI configured to monitor is more than 3 for the cell after applying the above steps

-	If the number of information bits in the DCI format 0_1 prior to padding is less than the payload size of the DCI format 1_1 for scheduling the same serving cell, a number of zero padding bits are generated for the DCI format 0_1 until the payload size equals that of the DCI format 1_1.

-	If the number of information bits in the DCI format 1_1 prior to padding is less than the payload size of the DCI format 0_1 for scheduling the same serving cell, zeros shall be appended to the DCI format 1_1 until the payload size equals that of the DCI format 0_1.

7.1.1.7.2.1.3	Test description

7.1.1.7.2.1.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that DRB is configured in RLC AM mode according to Table 7.1.1.7.2.1.3.1-1 and in addition NR Cell 3 is configured as NR Active SCell.

Table 7.1.1.7.2.1.3.1-1: RLC parameters

| t-PollRetransmit | ms80 |
| --- | --- |

7.1.1.7.2.1.3.2	Test procedure sequence

Table 7.1.1.7.2.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 |
| --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -85 | -85 |

Table 7.1.1.7.2.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message on NR SpCell (NR Cell 1) to configure NR SCell (NR Cell 3) and DCI format 1_3. (Note 1) | <-- | (RRCReconfiguration) | - | - |
| 2 | The UE transmits RRCReconfigurationComplete message. (Note 2) | --> | (RRCReconfigurationComplete) | - | - |
| 3 | The SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell. | <-- | DCI format 1_1 | - | - |
| 4 | The SS transmits Activation MAC control element to activate NR SCell on NR SpCell. | <-- | MAC PDU (SCell Activation/Deactivation MAC CE of one octet (C1=1)) | - | - |
| 5 | 200 ms after step 4, the SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell. The sCellDeactivationTimer is restarted in UE side. | <-- | DCI format 1_3 (only SCell is scheduled) | - | - |
| 6 | The SS transmits a MAC PDU (containing just padding or RLC status PDU, but no RLC data PDU) on NR SCell. | <-- | MAC PDU | - | - |
| 7 | 400 ms after step 4 the SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell. The sCellDeactivationTimer is restarted in UE side. | <-- | DCI format 1_3 (only SCell is scheduled) | - | - |
| 8 | The SS transmits a MAC PDU (containing an RLC PDU) on NR SCell. | <-- | MAC PDU | - | - |
| 9 | Check: Does the UE transmit a Scheduling Request on PUCCH on NR SpCell? | --> | (SR) | 1 | P |
| 10 | The SS sends an UL grant suitable for transmitting loop back PDU on NR SpCell. | <-- | (UL Grant) DCI format 0_1 | - | - |
| 11 | The UE transmits a MAC PDU containing the loop back PDU corresponding to step 8 on NR SpCell. | --> | MAC PDU | - | - |
| 12 | The SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell. | <-- | DCI format 1_3 (only SpCell is scheduled) | - | - |
| 13 | The SS transmits a MAC PDU containing RLC status PDU acknowledging reception of RLC PDU in step 11 on NR SpCell. | <-- | MAC PDU | - | - |
| 14 | 400 ms (sCellDeactivationTimer = 320 ms) after step 7, the SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell. | <-- | DCI format 1_3 (only SpCell is scheduled) | - | - |
| 15 | The SS transmits a MAC PDU (containing an RLC PDU) on NR SpCell. | <-- | MAC PDU | - | - |
| 16 | Check: Does the UE transmit a Scheduling Request on PUCCH? | --> | (SR) | 2 | P |
| 17 | The SS sends an UL grant suitable for transmitting loop back PDU on NR SpCell. | <-- | (UL Grant) DCI format 0_1 | - | - |
| 18 | The UE transmits a MAC PDU containing the loop back PDU corresponding to step 15 on NR SpCell. | --> | MAC PDU | - | - |
| 19 | The SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell. | <-- | DCI format 1_3 (only SpCell is scheduled) | - | - |
| 20 | The SS transmits a MAC PDU containing RLC status PDU acknowledging reception of RLC PDU in step 18 on NR SpCell. | <-- | MAC PDU | - | - |
| Note 1: Scheduling cell is NR SpCell (NR Cell 1), and reference cell is also NR SpCell (NR Cell 1). Therefore, the single DCI (DCI format 1_3) is counted on the NR Cell 1. Note 2: The NR RRCReconfigurationComplete message is scheduled by DCI format 0_0 to avoid DCI size misalignment for DCI format 0_1 (according to TS 38.212 [14] clause 7.3.1.0 step 4C) between UE and SS during the reconfiguration. |  |  |  |  |  |

7.1.1.7.2.1.3.3	Specific message contents

Table 7.1.1.7.2.1.3.3-1: RRCReconfiguration (step 1, Table 7.1.1.7.2.1.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| secondaryCellGroup | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.7.2.1.3.3-2: CellGroupConfig (Table 7.1.1.7.2.1.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| physicalCellGroupConfig | physicalCellGroupConfig | Table 7.1.1.7.2.1.3.3-5 |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated | ServingCellConfig | Table 7.1.1.7.2.1.3.3-3 |  |
| } |  |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SCellConfig { | 1 entry |  |  |
| SCellConfig[1] SEQUENCE { |  | entry 1 |  |
| sCellIndex | SCellIndex as per TS 38.508-1 [4] table 4.6.3-154 |  |  |
| sCellConfigCommon | ServingCellConfigCommon with condition No_UL and SCell_add |  |  |
| sCellConfigDedicated | ServingCellConfig | Table 7.1.1.7.2.1.3.3-6 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.7.2.1.3.3-3: ServingCellConfig (Table 7.1.1.7.2.1.3.3-2)

| Derivation Path: TS 38.508-1 [4],Table 4.6.3-167 with condition DCI 1_3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkDedicated | Table 7.1.1.7.2.1.3.3-4 |  |
| mc-DCI-SetOfCellsToAddModList-r18 SEQUENCE (SIZE (1..maxNrofSetsOfCells-r18)) OF MC-DCI-SetOfCells-r18 { | 1 entry |  |  |
| MC-DCI-SetOfCells-r18[1] SEQUENCE { |  |  |  |
| scheduledCellComboListDCI-1-3 | Not present |  | pc_coScheduledCellIndicationScheme_r18_cellInd_DCI_1_3=False AND pc_coScheduledCellIndicationScheme_r18_fdra_DCI_1_3=True |
| scheduledCellComboListDCI-1-3 SEQUENCE (SIZE (1..maxNrofCellCombos-r18)) OF ScheduledCellCombo-r18 { | 3 entries |  |  |
| ScheduledCellCombo-r18 [1] SEQUENCE (SIZE (1..maxNrofCellsInSet-r18)) OF INTEGER { |  | entry 1 |  |
| INTEGER[1] | 0 | The first entry of scheduledCellListDCI-1-3-r18 (i.e. SpCell) |  |
| } |  |  |  |
| ScheduledCellCombo-r18 [2] SEQUENCE (SIZE (1..maxNrofCellsInSet-r18)) OF INTEGER { |  | entry 2 |  |
| INTEGER[1] | 1 | The second entry of scheduledCellListDCI-1-3-r18 (i.e. SpCell) |  |
| } |  |  |  |
| ScheduledCellCombo-r18 [3] SEQUENCE (SIZE (1..maxNrofCellsInSet-r18)) OF INTEGER { |  | entry 3 |  |
| INTEGER[1] | 0 | The first entry of scheduledCellListDCI-1-3-r18 (i.e. SCell) |  |
| INTEGER[2] | 1 | The second entry of scheduledCellListDCI-1-3-r18 (i.e. SCell) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.7.2.1.3.3-4: BWP-DownlinkDedicated (Table 7.1.1.7.2.1.3.3-3)

| Derivation Path: TS 38.508-1 [4],Table 4.6.3-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkDedicated ::= SEQUENCE { |  |  |  |
| pdcch-Config CHOICE { |  |  |  |
| setup | PDCCH-Config with condition DCI_1_3 |  |  |
| } |  |  |  |
| pdsch-Config CHOICE { |  |  |  |
| setup | PDSCH-Config with condition DCI_1_3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.7.2.1.3.3-5: PhysicalCellGroupConfig (Table 7.1.1.7.2.1.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalCellGroupConfig ::= SEQUENCE { |  |  |  |
| pdsch-HARQ-ACK-Codebook | dynamic |  |  |
|  | semiStatic |  | pc_harqFeedbackType2_DCI_1_3=False AND pc_harqFeedbackType1_DCI_1_3=True |
| } |  |  |  |

Table 7.1.1.7.2.1.3.3-6: ServingCellConfig (Table 7.1.1.7.2.1.3.3-2)

| Derivation Path: TS 38.508-1 [4],Table 4.6.3-167 with condition No_UL and SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkDedicated | Table 7.1.1.7.2.1.3.3-7 |  |
| sCellDeactivationTimer | ms320 |  |  |
| } |  |  |  |

Table 7.1.1.7.2.1.3.3-7: BWP-DownlinkDedicated (Table 7.1.1.7.2.1.3.3-6)

| Derivation Path: TS 38.508-1 [4],Table 4.6.3-11 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkDedicated ::= SEQUENCE { |  |  |  |
| pdcch-Config | Not present |  |  |
| pdsch-Config CHOICE { |  |  |  |
| setup | PDSCH-Config with condition DCI_1_3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.7.2.1.3.3-8: Physical layer parameters for DCI format 1_3 (steps 5, 7, 12, 14 and 19 for scheduledCellComboListDCI-1-3 present configuration, Table 7.1.1.7.2.1.3.2-2)

| Derivation path: 38.508-1 [4] Table 4.3.6.1.2.4-1 with condition CellInd |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| Scheduled cells indicator | The first entry of scheduledCellComboListDCI-1-3 | “00” | Step 12,14,19 |
|  | The second entry of scheduledCellComboListDCI-1-3 | “01” | Step 5,7 |
| Frequency domain resource assignment | block number 1(SpCell): dependent on the Downlink TBS specified in 38.523-3 Annex B.1 Note: NcellDL=1 | - | Step 12,14,19 |
|  | block number 1(SCell) : dependent on the Downlink TBS specified in 38.523-3 Annex B.1 Note: NcellDL=1 | - | Step 5,7 |

Table 7.1.1.7.2.1.3.3-9: Physical layer parameters for DCI format 1_3 (steps 5, 7, 12, 14 and 19 for scheduledCellComboListDCI-1-3 notpresent configuration, Table 7.1.1.7.2.1.3.2-2)

| Derivation path: TS 38.508-1 [4] Table 4.3.6.1.2.4-1 with condition FDRA and DL_CELL2 |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| Frequency domain resource assignment | block number 1(SpCell): dependent on the Downlink TBS specified in 38.523-3 Annex B.1 block number 2(SCell) : all bits set to 1 Note: NcellDL=2 | - | Step 12,14,19 |
|  | block number 1(SpCell): all bits set to 1 block number 2(SCell) : dependent on the Downlink TBS specified in 38.523-3 Annex B.1 Note: NcellDL=2 | - | Step 5,7 |

###### 7.1.1.7.2.2	Activation/Deactivation of SCells / Multi-cell PDSCH scheduling with a single DCI / Search space for single DCI is only configured in scheduling cell / sCellDeactivationTimer / Inter-Band CA

The scope and description of the present TC is the same as test case 7.1.1.7.2.1 with the following differences:

-	CA configuration: Inter-band CA replaces Intra-band Contiguous CA

-	Cells configuration: NR Cell 10 replaces NR Cell 3

- 	NR Cell 1 and NR Cell 10 have same SCS/carrier type.

###### 7.1.1.7.2.3	Activation/Deactivation of SCells / Multi-cell PDSCH scheduling with a single DCI / Search space for single DCI is only configured in scheduling cell / sCellDeactivationTimer / Intra-band non-Contiguous CA

The scope and description of the present TC is the same as test case 7.1.1.7.2.1 with the following differences:

-	CA configuration: Intra-band non-Contiguous CA replaces Intra-band Contiguous CA

##### 7.1.1.7.3	Activation/Deactivation of SCells / Multi-cell PUSCH scheduling with a single DCI / Search space for single DCI is only configured in scheduling cell / sCellDeactivationTimer

###### 7.1.1.7.3.1	Activation/Deactivation of SCells / Multi-cell PUSCH scheduling with a single DCI / Search space for single DCI is only configured in scheduling cell / sCellDeactivationTimer / Intra-band Contiguous CA

7.1.1.7.3.1.1	Test Purpose (TP)

(1)

with( UE in RRC_CONNECTED state with SCell activated, and the search space for single DCI is only configured in scheduling cell (i.e. the Reference cell is scheduling cell) )

ensure that {

when{ UE receives a DCI format 0-3 on NR SpCell PDCCH to schedule SCell }

then {  UE restarts the sCellDeactivationTimer }

}

(2)

with ( UE in RRC_CONNECTED state with SCell activated, and the search space for single DCI is only configured in scheduling cell (i.e. the Reference cell is scheduling cell))

ensure that {

when{ UE sCellDeactivationTimer expires }

then { UE does not stop monitoring PDCCH for scheduling multiple cells for the set of cells in MC-DCI-SetOfCells including the SCell (i.e. does not stop monitoring DCI format 0_3 on the scheduling cell for the set of cells) }

}

7.1.1.7.3.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.321, clauses 5.9 and TS 38.212, clauses 7.3.1.0. Unless otherwise stated these are Rel-18 requirements.

[TS 38.321, clause 5.9]

If the MAC entity is configured with one or more SCells, the network may activate and deactivate the configured SCells. Upon configuration of an SCell, the SCell is deactivated unless the parameter sCellState is set to activated for the SCell by upper layers.

The configured SCell(s) is activated and deactivated by:

-	receiving the SCell Activation/Deactivation MAC CE described in clause 6.1.3.10;

-	receiving the Enhanced SCell Activation/Deactivation MAC CE described in clause 6.1.3.55;

-	configuring sCellDeactivationTimer timer per configured SCell (except the SCell configured with PUCCH, if any): the associated SCell is deactivated upon its expiry;

-	configuring sCellState per configured SCell: if configured, the associated SCell is activated upon SCell configuration;

-	receiving scg-State: the SCells of SCG are deactivated.

The MAC entity shall for each configured SCell:

1>	if an SCell is configured with sCellState set to activated upon SCell configuration, or an SCell Activation/Deactivation MAC CE or an Enhanced SCell Activation/Deactivation MAC CE is received activating the SCell:

…

1>	else if an SCell Activation/Deactivation MAC CE or an Enhanced SCell Activation/Deactivation MAC CE is received deactivating the SCell; or

1>	if the sCellDeactivationTimer associated with the activated SCell expires; or

1>	if the SCG associated with the activated SCell is deactivated:

2>	deactivate the SCell according to the timing defined in TS 38.213 [6];

2>	stop the sCellDeactivationTimer associated with the SCell;

2>	stop the bwp-InactivityTimer associated with the SCell;

2>	deactivate any active BWP associated with the SCell;

2>	clear any configured downlink assignment and any configured uplink grant Type 2 associated with the SCell respectively;

2>	clear any PUSCH resource for semi-persistent CSI reporting associated with the SCell;

2>	suspend any configured uplink grant Type 1 associated with the SCell;

2>	flush all HARQ buffers associated with the SCell;

2>	cancel, if any, triggered consistent LBT failure for the SCell.

1>	if PDCCH on the activated SCell indicates an uplink grant or downlink assignment; or

1>	if PDCCH on the Serving Cell scheduling the activated SCell indicates an uplink grant or a downlink assignment for the activated SCell; or

1>	if a MAC PDU is transmitted in a configured uplink grant and LBT failure indication is not received from lower layers; or

1>	if a MAC PDU is received in a configured downlink assignment:

2>	restart the sCellDeactivationTimer associated with the SCell.

1>	if the SCell is deactivated:

2>	not transmit SRS on the SCell;

2>	not report CSI for the SCell;

2>	not transmit on UL-SCH on the SCell;

2>	not transmit on RACH on the SCell;

2>	not monitor the PDCCH on the SCell;

2>	not monitor the PDCCH for the SCell;

2>	not transmit PUCCH on the SCell;

2>	if the SCell is configured as a scheduled cell in MC-DCI-SetOfCells and with the search space for DCI to schedule multiple cells (as specified in TS 38.213 [6]) of the same searchSpaceId as the serving cell in which MC-DCI-SetOfCells containing the SCell is configured:

3>	not monitor the PDCCH for scheduling multiple cells (as specified in TS 38.213 [6]) for the set of cells in MC-DCI-SetOfCells including the SCell.

[TS 38.212, clause 7.3.1.0]

Step 3:

-	If both of the following conditions are fulfilled the size alignment procedure is complete

-	the total number of different DCI sizes configured to monitor is no more than 4 for the cell

-	the total number of different DCI sizes with C-RNTI configured to monitor is no more than 3 for the cell

Step 4:

-	Otherwise

Step 4A:

…

Step 4B:

…

Step 4C:

-	If the total number of different DCI sizes configured to monitor is more than 4 for the cell after applying the above steps, or if the total number of different DCI sizes with C-RNTI configured to monitor is more than 3 for the cell after applying the above steps

-	If the number of information bits in the DCI format 0_1 prior to padding is less than the payload size of the DCI format 1_1 for scheduling the same serving cell, a number of zero padding bits are generated for the DCI format 0_1 until the payload size equals that of the DCI format 1_1.

-	If the number of information bits in the DCI format 1_1 prior to padding is less than the payload size of the DCI format 0_1 for scheduling the same serving cell, zeros shall be appended to the DCI format 1_1 until the payload size equals that of the DCI format 0_1.

Step 4D:

-	If the total number of different DCI sizes configured to monitor is more than 4 for the cell after applying the above steps and the cell is the serving cell for counting the size of one or both DCI format 0_3 and DCI format 1_3 as defined in Clause 10.1 of [5, TS38.213], or if the total number of different DCI sizes with C-RNTI configured to monitor is more than 3 for the cell after applying the above steps and the cell is the serving cell for counting the size of one or both DCI format 0_3 and DCI format 1_3 as defined in Clause 10.1 of [5, TS38.213]

-	If the number of information bits in the DCI format 0_3 prior to padding is less than the payload size of the DCI format 1_3 for scheduling the same cell set, a number of zero padding bits are generated for the DCI format 0_3 until the payload size equals that of the DCI format 1_3.

-	If the number of information bits in the DCI format 1_3 prior to padding is less than the payload size of the DCI format 0_3 for scheduling the same cell set, zeros shall be appended to the DCI format 1_3 until the payload size equals that of the DCI format 0_3.

7.1.1.7.3.1.3	Test description

7.1.1.7.3.1.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that DRB is configured in RLC AM mode according to Table 7.1.1.7.3.1.3.1-1 and in addition NR Cell 3 is configured as NR Active SCell.

Table 7.1.1.7.3.1.3.1-1: RLC parameters

| t-PollRetransmit | ms80 |
| --- | --- |

7.1.1.7.3.1.3.2	Test procedure sequence

Table 7.1.1.7.3.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 |
| --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -85 | -85 |

Table 7.1.1.7.3.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message on NR SpCell (NR Cell 1) to configure NR SCell (NR Cell 3) and DCI format 1_3 and/or DCI format 0_3. (Note 1) | <-- | RRCReconfiguration | - | - |
| 2 | The UE transmits RRCReconfigurationComplete message. (Note 2) | --> | RRCReconfigurationComplete | - | - |
| 3 | The SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell. | <-- | DCI format 1_1 | - | - |
| 4 | The SS transmits Activation MAC control element to activate NR SCell on NR SpCell. | <-- | MAC PDU (SCell Activation/Deactivation MAC CE of one octet (C1=1)) | - | - |
| 5 | 200 ms after step 4, the SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell. (Note 3) | <-- | DCI format 1_1 or DCI format 1_3 (only SpCell is scheduled) | - | - |
| 6 | The SS transmits a MAC PDU (containing an RLC PDU) on NR SpCell. | <-- | MAC PDU | - | - |
| 7 | The UE transmit a Scheduling Request on PUCCH on NR SpCell. | --> | (SR) | - | - |
| 8 | The SS sends an UL grant suitable for transmitting loop back PDU on NR SpCell. The sCellDeactivationTimer is restarted in UE side. | <-- | (UL Grant) DCI format 0_3 (only SCell is scheduled) | - | - |
| 9 | The UE transmits a MAC PDU containing the loop back PDU corresponding to step 6 on NR SCell. | --> | MAC PDU | - | - |
| 10 | The SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell. (Note 3) | <-- | DCI format 1_1 or DCI format 1_3 (only SpCell is scheduled) | - | - |
| 11 | The SS transmits a MAC PDU containing RLC status PDU acknowledging reception of RLC PDU in step 9 on NR SpCell. | <-- | MAC PDU | - | - |
| 12 | 400 ms after step 4 the SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell. (Note 3) | <-- | DCI format 1_1 or DCI format 1_3 (only SpCell is scheduled) | - | - |
| 13 | The SS transmits a MAC PDU (containing an RLC PDU) on NR SpCell. | <-- | MAC PDU | - | - |
| 14 | The UE transmit a Scheduling Request on PUCCH on NR SpCell | --> | (SR) | - | - |
| 15 | The SS sends an UL grant suitable for transmitting loop back PDU on NR SpCell. The sCellDeactivationTimer is restarted in UE side. | <-- | (UL Grant) DCI format 0_3 (only SCell is scheduled) | - | - |
| 16 | The UE transmits a MAC PDU containing the loop back PDU corresponding to step 13 on NR SCell. | --> | MAC PDU | 1 | P |
| 17 | The SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell. (Note 3) | <-- | DCI format 1_1 or DCI format 1_3 (only SpCell is scheduled) | - | - |
| 18 | The SS transmits a MAC PDU containing RLC status PDU acknowledging reception of RLC PDU in step 16 on NR SpCell. | <-- | MAC PDU | - | - |
| 19 | 400 ms (sCellDeactivationTimer = 320 ms) after step 15, the SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell. (Note 3) | <-- | DCI format 1_1 or DCI format 1_3 (only SpCell is scheduled) | - | - |
| 20 | The SS transmits a MAC PDU (containing an RLC PDU) on NR SpCell. | <-- | MAC PDU | - | - |
| 21 | The UE transmit a Scheduling Request on PUCCH | --> | (SR) | - | - |
| 22 | The SS sends an UL grant suitable for transmitting loop back PDU on NR SpCell. | <-- | (UL Grant) DCI format 0_3 (only SpCell is scheduled) | - | - |
| 23 | The UE transmits a MAC PDU containing the loop back PDU corresponding to step 20 on NR SpCell. | --> | MAC PDU | 2 | P |
| 24 | The SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell. (Note 3) | <-- | DCI format 1_1 or DCI format 1_3 (only SpCell is scheduled) | - | - |
| 25 | The SS transmits a MAC PDU containing RLC status PDU acknowledging reception of RLC PDU in step 25 on NR SpCell. | <-- | MAC PDU | - | - |
| Note 1: Scheduling cell is NR SpCell (NR Cell 1), and reference cell is also NR SpCell (NR Cell 1). Therefore, the single DCI (DCI format 0_3/1_3) is counted on the NR Cell 1, and DCI size alignment procedure specified inTS 38.212 [14] clause 7.3.1.0 for NR Cell 1 is complete in step 4C or step 4D. Note 2: For SA the NR RRCReconfigurationComplete message is scheduled by DCI format 0_0 to avoid DCI size misalignment for DCI format 0_1 (according to TS 38.212 [14] clause 7.3.1.0 step 4C) between UE and SS during the reconfiguration. Note 3: If the pc_multiCell_PUSCH_DCI_1_3_SameSCS_r18 = True, downlink assignment is DCI format 1_3, else the downlink assignment is DCI format 1_1. |  |  |  |  |  |

7.1.1.7.3.1.3.3	Specific message contents

Table 7.1.1.7.3.1.3.3-1: RRCReconfiguration (step 1, Table 7.1.1.7.3.1.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| secondaryCellGroup | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.7.3.1.3.3-2: CellGroupConfig (Table 7.1.1.7.3.1.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| physicalCellGroupConfig | physicalCellGroupConfig | Table 7.1.1.7.3.1.3.3-6 | pc_multiCell_PDSCH_DCI_1_3_SameSCS_r18 |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated | ServingCellConfig | Table 7.1.1.7.3.1.3.3-3 |  |
| } |  |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SCellConfig { | 1 entry |  |  |
| SCellConfig[1] SEQUENCE { |  | entry 1 |  |
| sCellIndex | SCellIndex as per TS 38.508-1 [4] table 4.6.3-154 |  |  |
| sCellConfigCommon | ServingCellConfigCommon with condition SCell_add |  |  |
| sCellConfigDedicated | ServingCellConfig | Table 7.1.1.7.3.1.3.3-7 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.7.3.1.3.3-3: ServingCellConfig (Table 7.1.1.7.3.1.3.3-2)

| Derivation Path: TS 38.508-1 [4],Table 4.6.3-167 with condition (DCI_0_3) OR (DCI_0_3 and DCI_1_3) (NOTE 1) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkDedicated | Table 7.1.1.7.3.1.3.3-4 |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkDedicated | Table 7.1.1.7.3.1.3.3-5 |  |
| } |  |  |  |
| mc-DCI-SetOfCellsToAddModList-r18 SEQUENCE (SIZE (1..maxNrofSetsOfCells-r18)) OF MC-DCI-SetOfCells-r18 { | 1 entry |  |  |
| MC-DCI-SetOfCells-r18[1] SEQUENCE { |  |  |  |
| scheduledCellComboListDCI-1-3 | Not present |  | pc_coScheduledCellIndicationScheme_r18_cellInd_DCI_1_3=False AND pc_coScheduledCellIndicationScheme_r18_fdra_DCI_1_3=True |
| scheduledCellComboListDCI-1-3 SEQUENCE (SIZE (1..maxNrofCellCombos-r18)) OF ScheduledCellCombo-r18 { | 3 entries |  |  |
| ScheduledCellCombo-r18 [1] SEQUENCE (SIZE (1..maxNrofCellsInSet-r18)) OF INTEGER { |  | entry 1 |  |
| INTEGER[1] | 0 | The first entry of scheduledCellListDCI-1-3-r18 (i.e. SpCell) |  |
| } |  |  |  |
| ScheduledCellCombo-r18 [2] SEQUENCE (SIZE (1..maxNrofCellsInSet-r18)) OF INTEGER { |  | entry 2 |  |
| INTEGER[1] | 1 | The second entry of scheduledCellListDCI-1-3-r18 (i.e. SCell) |  |
| } |  |  |  |
| ScheduledCellCombo-r18 [3] SEQUENCE (SIZE (1..maxNrofCellsInSet-r18)) OF INTEGER { |  | entry 3 |  |
| INTEGER[1] | 0 | The first entry of scheduledCellListDCI-1-3-r18 (i.e. SpCell) |  |
| INTEGER[2] | 1 | The second entry of scheduledCellListDCI-1-3-r18 (i.e. SCell) |  |
| } |  |  |  |
| } |  |  |  |
| scheduledCellComboListDCI-0-3-r18 | Not present |  | pc_coScheduledCellIndicationScheme_r18_fdra_DCI_0_3=True AND pc_coScheduledCellIndicationScheme_r18_cellInd_DCI_0_3=False |
| scheduledCellComboListDCI-0-3-r18 SEQUENCE (SIZE (1..maxNrofCellCombos-r18)) OF ScheduledCellCombo-r18 { | 3 entries |  |  |
| ScheduledCellCombo-r18 [1] SEQUENCE (SIZE (1..maxNrofCellsInSet-r18)) OF INTEGER { |  | entry 1 |  |
| INTEGER[1] | 0 | The first entry of scheduledCellListDCI-0-3-r18 (i.e. SpCell) |  |
| } |  |  |  |
| ScheduledCellCombo-r18 [2] SEQUENCE (SIZE (1..maxNrofCellsInSet-r18)) OF INTEGER { |  | entry 2 |  |
| INTEGER[1] | 1 | The second entry of scheduledCellListDCI-0-3-r18 (i.e. SCell) |  |
| } |  |  |  |
| ScheduledCellCombo-r18 [3] SEQUENCE (SIZE (1..maxNrofCellsInSet-r18)) OF INTEGER { |  | entry 3 |  |
| INTEGER[1] | 0 | The first entry of scheduledCellListDCI-0-3-r18 (i.e. SpCell) |  |
| INTEGER[2] | 1 | The second entry of scheduledCellListDCI-0-3-r18 (i.e. SCell) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| Note 1: If the pc_multiCell_PDSCH_DCI_1_3_SameSCS_r18=True, the condition is DCI_0_3 and DCI_1_3, else, the condition is DCI_0_3. |  |  |  |

Table 7.1.1.7.3.1.3.3-4: BWP-DownlinkDedicated (Table 7.1.1.7.3.1.3.3-3)

| Derivation Path: TS 38.508-1 [4],Table 4.6.3-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkDedicated ::= SEQUENCE { |  |  |  |
| pdcch-Config CHOICE { |  |  |  |
| setup | PDCCH-Config with condition DCI_0_3 |  |  |
|  | PDCCH-Config with condition DCI_0_3 and DCI_1_3 |  | pc_multiCell_PDSCH_DCI_1_3_SameSCS_r18 |
| } |  |  |  |
| pdsch-Config CHOICE { |  |  | pc_multiCell_PDSCH_DCI_1_3_SameSCS_r18 |
| setup | PDSCH-Config with condition DCI_1_3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.7.3.1.3.3-5: BWP-UplinkDedicated (Table 7.1.1.7.3.1.3.3-3)

| Derivation Path: TS 38.508-1 [4],Table 4.6.3-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicated ::= SEQUENCE { |  |  |  |
| pusch-Config CHOICE { |  |  |  |
| setup | PUSCH-Config with condition DCI_0_3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.7.3.1.3.3-6: PhysicalCellGroupConfig (Table 7.1.1.7.3.1.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalCellGroupConfig ::= SEQUENCE { |  |  |  |
| pdsch-HARQ-ACK-Codebook | dynamic |  | pc_harqFeedbackType2_DCI_1_3=True AND pc_harqFeedbackType1_DCI_1_3=False |
|  | semiStatic |  |  |
| } |  |  |  |

Table 7.1.1.7.3.1.3.3-7: ServingCellConfig (Table 7.1.1.7.3.1.3.3-2)

| Derivation Path: TS 38.508-1 [4],Table 4.6.3-167 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkDedicated | Table 7.1.1.7.3.1.3.3-8 |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkDedicated | Table 7.1.1.7.3.1.3.3-9 |  |
| } |  |  |  |
| sCellDeactivationTimer | ms320 |  |  |
| } |  |  |  |

Table 7.1.1.7.3.1.3.3-8: BWP-DownlinkDedicated (Table 7.1.1.7.3.1.3.3-7)

| Derivation Path: TS 38.508-1 [4],Table 4.6.3-11 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkDedicated ::= SEQUENCE { |  |  |  |
| pdcch-Config | Not present |  |  |
| pdsch-Config CHOICE { |  |  | pc_multiCell_PDSCH_DCI_1_3_SameSCS_r18 |
| setup | PDSCH-Config with condition DCI_1_3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.7.3.1.3.3-9: BWP-UplinkDedicated (Table 7.1.1.7.3.1.3.3-7)

| Derivation Path: TS 38.508-1 [4],Table 4.6.3-15 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicated ::= SEQUENCE { |  |  |  |
| pusch-Config CHOICE { |  |  |  |
| setup | PUSCH-Config with condition DCI_0_3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.7.3.1.3.3-10: Physical layer parameters for DCI format 1_3 (steps 5, 10, 12, 17, 19 and 24 for scheduledCellComboListDCI-1-3 present configuration, Table 7.1.1.7.3.1.3.2-2)

| Derivation path: 38.508-1 [4] Table 4.3.6.1.2.4-1 with condition CellInd |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| Scheduled cells indicator | The first entry of scheduledCellComboListDCI-1-3 | “00” |  |
| Frequency domain resource assignment | block number 1(SpCell): dependent on the Downlink TBS specified in TS 38.523-3 [3] Annex B.1 Note: NcellDL=1 | - |  |

Table 7.1.1.7.3.1.3.3-11: Physical layer parameters for DCI format 1_3 (steps 5, 10, 12, 17, 19 and 24 for scheduledCellComboListDCI-1-3 not present configuration, Table 7.1.1.7.3.1.3.2-2)

| Derivation path: TS 38.508-1 [4] Table 4.3.6.1.2.4-1 with condition FDRA and DL_CELL2 |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| Frequency domain resource assignment | block number 1(SpCell): dependent on the Downlink TBS specified in TS 38.523-3 [3] Annex B.1 block number 2(SCell) : all bits set to 1 Note: NcellDL=2 | - |  |

Table 7.1.1.7.3.1.3.3-12: Physical layer parameters for DCI format 0_3 (steps 8, 15 and 22 for scheduledCellComboListDCI-0-3 present configuration, Table 7.1.1.7.3.1.3.2-2)

| Derivation path: 38.508-1 [4] Table 4.3.6.1.2.4-1 with condition CellInd |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| Scheduled cells indicator | The first entry of scheduledCellComboListDCI-1-3 | “00” | Step 22 |
|  | The second entry of scheduledCellComboListDCI-1-3 | “01” | Step 8,15 |
| Frequency domain resource assignment | block number 1(SpCell): dependent on the Downlink TBS specified in TS 38.523-3 [3] Annex B.1 Note: NcellDL=1 | - | Step 22 |
|  | block number 1(SCell) : dependent on the Downlink TBS specified in TS 38.523-3 [3] Annex B.1 Note: NcellDL=1 | - | Step 8,15 |

Table 7.1.1.7.3.1.3.3-13: Physical layer parameters for DCI format 0_3 (steps 8, 15 and 22 for scheduledCellComboListDCI-0-3 not present configuration, Table 7.1.1.7.3.1.3.2-2)

| Derivation path: TS 38.508-1 [4] Table 4.3.6.1.2.4-1 with condition FDRA and DL_CELL2 |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| Frequency domain resource assignment | block number 1(SpCell): dependent on the Downlink TBS specified in TS 38.523-3 [3] Annex B.1 block number 2(SCell) : all bits set to 1 Note: NcellDL=2 | - | Step 22 |
|  | block number 1(SpCell): all bits set to 1 block number 2(SCell) : dependent on the Downlink TBS specified in TS 38.523-3 [3] Annex B.1 Note: NcellDL=2 | - | Step 8,15 |

###### 7.1.1.7.3.2	Activation/Deactivation of SCells / Multi-cell PUSCH scheduling with a single DCI / Search space for single DCI is only configured in scheduling cell / sCellDeactivationTimer / Inter-Band CA

The scope and description of the present TC is the same as test case 7.1.1.7.3.1 with the following differences:

-	CA configuration: Inter-band CA replaces Intra-band Contiguous CA

-	Cells configuration: NR Cell 10 replaces NR Cell 3

- 	NR Cell 1 and NR Cell 10 have same SCS/carrier type.

###### 7.1.1.7.3.3	Activation/Deactivation of SCells / Multi-cell PUSCH scheduling with a single DCI / Search space for single DCI is only configured in scheduling cell / sCellDeactivationTimer / Intra-band non-Contiguous CA

The scope and description of the present TC is the same as test case 7.1.1.7.3.1 with the following differences:

-	CA configuration: Intra-band non-Contiguous CA replaces Intra-band Contiguous CA

##### 7.1.1.7.4	Activation/Deactivation of SCells / Multi-cell PDSCH scheduling with a single DCI / Search space for single DCI is not only configured in scheduling cell / sCellDeactivationTimer

###### 7.1.1.7.4.1	Activation/Deactivation of SCells / Multi-cell PDSCH scheduling with a single DCI / Search space for single DCI is not only configured in scheduling cell / sCellDeactivationTimer / Intra-band Contiguous CA

7.1.1.7.4.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with SCell configured, SCell is configured as a scheduled cell in MC-DCI-SetOfCells and with the search space for DCI to schedule multiple cells of the same searchSpaceId as the serving cell in which MC-DCI-SetOfCells containing the SCell is configured( i.e. the reference cell is SCell ) }

ensure that {

when { UE receives an SCell Activation/Deactivation MAC CE activating the Scell and sCellDeactivationTimer does not expire }

then { UE monitors PDCCH for scheduling multiple cells for the set of cells in MC-DCI-SetOfCells including the SCell(i.e. monitors DCI format 1_3 on the scheduling cell for the set of cells) }

}

(2)

with( UE in RRC_CONNECTED state with SCell activated, SCell is configured as a scheduled cell in MC-DCI-SetOfCells and with the search space for DCI to schedule multiple cells of the same searchSpaceId as the serving cell in which MC-DCI-SetOfCells containing the SCell is configured( i.e. the reference cell is SCell )  )

ensure that {

when{ UE receives a DCI format 1-3 on NR SpCell PDCCH to schedule SCell }

then {  UE restarts the sCellDeactivationTimer }

}

(3)

with ( UE in RRC_CONNECTED state with SCell activated, SCell is configured as a scheduled cell in MC-DCI-SetOfCells and with the search space for DCI to schedule multiple cells of the same searchSpaceId as the serving cell in which MC-DCI-SetOfCells containing the SCell is configured( i.e. the reference cell is SCell ))

ensure that {

when{ UE sCellDeactivationTimer expires }

then { UE deactivates the SCell and stops monitoring PDCCH for scheduling multiple cells for the set of cells in MC-DCI-SetOfCells including the SCell (i.e. stops monitoring DCI format 1_3 on the scheduling cell for the set of cells) }

}

7.1.1.7.4.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.321, clause 5.9, TS 38.212, clause 7.3.1.0 and TS 38.213, clause 10.1. Unless otherwise stated these are Rel-18 requirements.

[TS 38.321, clause 5.9]

If the MAC entity is configured with one or more SCells, the network may activate and deactivate the configured SCells. Upon configuration of an SCell, the SCell is deactivated unless the parameter sCellState is set to activated for the SCell by upper layers.

The configured SCell(s) is activated and deactivated by:

-	receiving the SCell Activation/Deactivation MAC CE described in clause 6.1.3.10;

-	receiving the Enhanced SCell Activation/Deactivation MAC CE described in clause 6.1.3.55;

-	configuring sCellDeactivationTimer timer per configured SCell (except the SCell configured with PUCCH, if any): the associated SCell is deactivated upon its expiry;

-	configuring sCellState per configured SCell: if configured, the associated SCell is activated upon SCell configuration;

-	receiving scg-State: the SCells of SCG are deactivated.

The MAC entity shall for each configured SCell:

1>	if an SCell is configured with sCellState set to activated upon SCell configuration, or an SCell Activation/Deactivation MAC CE or an Enhanced SCell Activation/Deactivation MAC CE is received activating the SCell:

…

2>	if the SCell was deactivated prior to receiving this SCell Activation/Deactivation MAC CE or this Enhanced SCell Activation/Deactivation MAC CE; or

2>	if the SCell is configured with sCellState set to activated upon SCell configuration:

3>	if firstActiveDownlinkBWP-Id is not set to dormant BWP:

4>	activate the SCell according to the timing defined in TS 38.213 [6] for MAC CE activation and according to the timing defined in TS 38.133 [11] for direct SCell activation; i.e. apply normal SCell operation including:

5>	SRS transmissions on the SCell;

5>	CSI reporting for the SCell;

5>	PDCCH monitoring on the SCell;

5>	PDCCH monitoring for the SCell;

5>	PUCCH transmissions on the SCell, if configured.

…

3>	activate the DL BWP and UL BWP indicated by firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id respectively.

2>	start or restart the sCellDeactivationTimer associated with the SCell according to the timing defined in TS 38.213 [6] for MAC CE activation and according to the timing defined in TS 38.133 [11] for direct SCell activation;

…

1>	else if an SCell Activation/Deactivation MAC CE or an Enhanced SCell Activation/Deactivation MAC CE is received deactivating the SCell; or

1>	if the sCellDeactivationTimer associated with the activated SCell expires; or

1>	if the SCG associated with the activated SCell is deactivated:

2>	deactivate the SCell according to the timing defined in TS 38.213 [6];

2>	stop the sCellDeactivationTimer associated with the SCell;

2>	stop the bwp-InactivityTimer associated with the SCell;

2>	deactivate any active BWP associated with the SCell;

2>	clear any configured downlink assignment and any configured uplink grant Type 2 associated with the SCell respectively;

2>	clear any PUSCH resource for semi-persistent CSI reporting associated with the SCell;

2>	suspend any configured uplink grant Type 1 associated with the SCell;

2>	flush all HARQ buffers associated with the SCell;

2>	cancel, if any, triggered consistent LBT failure for the SCell.

1>	if PDCCH on the activated SCell indicates an uplink grant or downlink assignment; or

1>	if PDCCH on the Serving Cell scheduling the activated SCell indicates an uplink grant or a downlink assignment for the activated SCell; or

1>	if a MAC PDU is transmitted in a configured uplink grant and LBT failure indication is not received from lower layers; or

1>	if a MAC PDU is received in a configured downlink assignment:

2>	restart the sCellDeactivationTimer associated with the SCell.

1>	if the SCell is deactivated:

2>	not transmit SRS on the SCell;

2>	not report CSI for the SCell;

2>	not transmit on UL-SCH on the SCell;

2>	not transmit on RACH on the SCell;

2>	not monitor the PDCCH on the SCell;

2>	not monitor the PDCCH for the SCell;

2>	not transmit PUCCH on the SCell;

2>	if the SCell is configured as a scheduled cell in MC-DCI-SetOfCells and with the search space for DCI to schedule multiple cells (as specified in TS 38.213 [6]) of the same searchSpaceId as the serving cell in which MC-DCI-SetOfCells containing the SCell is configured:

3>	not monitor the PDCCH for scheduling multiple cells (as specified in TS 38.213 [6]) for the set of cells in MC-DCI-SetOfCells including the SCell.

[TS 38.212, clause 7.3.1.0]

Step 3:

-	If both of the following conditions are fulfilled the size alignment procedure is complete

-	the total number of different DCI sizes configured to monitor is no more than 4 for the cell

-	the total number of different DCI sizes with C-RNTI configured to monitor is no more than 3 for the cell

[TS 38.213, clause 10.1]

A UE expects to monitor PDCCH candidates for up to 4 sizes of DCI formats that include up to 3 sizes of DCI formats with CRC scrambled by C-RNTI per serving cell. The UE counts a number of sizes for DCI formats per serving cell based on a number of configured PDCCH candidates in respective search space sets for the corresponding active DL BWP. If the UE monitors PDCCH candidates for detection of one or both of DCI format 0_3 and DCI format 1_3 for scheduling on serving cells from a set of serving cells, the serving cell for counting the size of one or both DCI format 0_3 and DCI format 1_3, respectively, is

-	the scheduling cell, if the scheduling cell is included in the set of serving cells and the UE is provided search space sets for the PDCCH candidates only on the scheduling cell

-	a serving cell from the set of serving cells, if search space sets with same searchSpaceId for one or both of DCI format 0_3 and DCI format 1_3, respectively, are provided on the serving cell and on the scheduling cell.

…

In the following, if a UE monitors PDCCH candidates on a scheduling cell for detection of DCI format 0_3 or DCI format 1_3 for scheduling on serving cells from a set of serving cells, the serving cell for counting the PDCCH candidates and a corresponding number of non-overlapping CCEs is

-	the scheduling cell, if the scheduling cell is included in the set of serving cells and the UE is provided search space sets for the PDCCH candidates only on the scheduling cell

-	a serving cell from the set of serving cells, if search space sets with same searchSpaceId for one or both of DCI format 0_3 and DCI format 1_3, respectively, are provided on the serving cell and on the scheduling cell.

[TS 38.331, clause 6.3.2]

dci-FormatsMC

Indicates whether the UE monitors in this USS for DCI format 0_3 or for format 1_3 or for formats 0_3 and 1_3. Separate search space sets for DCI formats in this field and the DCI formats in ue-Specific (without suffix) are independently configured.

NOTE:	This parameter is used only for SearchSpace configured to the scheduling cell, while another SearchSpace configured to the reference scheduled cell (if any) configures only nrofCandidates (i.e., all other optional fields are absent) with same searchSpaceId with that for scheduling cell.

7.1.1.7.4.1.3	Test description

7.1.1.7.4.1.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that DRB is configured in RLC AM mode according to Table 7.1.1.7.4.1.3.1-1 and in addition NR Cell 3 is configured as NR Active SCell.

Table 7.1.1.7.4.1.3.1-1: RLC parameters

| t-PollRetransmit | ms80 |
| --- | --- |

7.1.1.7.4.1.3.2	Test procedure sequence

Table 7.1.1.7.4.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 |
| --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -85 | -85 |

Table 7.1.1.7.4.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits an RRCReconfiguration message on NR SpCell (NR Cell 1) to configure NR SCell (NR Cell 3) and DCI format 1_3. (Note 1) | <-- | RRCReconfiguration | - | - |
| 2 | The UE transmits RRCReconfigurationComplete message. | --> | RRCReconfigurationComplete | - | - |
| 3 | The SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell. | <-- | DCI format 1_1 | - | - |
| 4 | The SS transmits Activation MAC control element to activate NR SCell on NR SpCell. | <-- | MAC PDU (SCell Activation/Deactivation MAC CE of one octet (C1=1)) | - | - |
| 5 | 200 ms after step 4, the SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell. The sCellDeactivationTimer is restarted in UE side. | <-- | DCI format 1_3 (only SCell is scheduled) | - | - |
| 6 | The SS transmits a MAC PDU (containing an RLC PDU) on NR SCell. | <-- | MAC PDU | - | - |
| 7 | Check: Does the UE transmit a Scheduling Request on PUCCH? | --> | (SR) | 1 | P |
| 8 | The SS sends an UL grant suitable for transmitting loop back PDU on NR SpCell. | <-- | (UL Grant) DCI format 0_1 | - | - |
| 9 | The UE transmits a MAC PDU containing the loop back PDU corresponding to step 6 on NR SpCell. | --> | MAC PDU | - | - |
| 10 | The SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell. | <-- | DCI format 1_3 (only SpCell is scheduled) | - | - |
| 11 | The SS transmits a MAC PDU containing RLC status PDU acknowledging reception of RLC PDU in step 9 on NR SpCell. | <-- | MAC PDU | - | - |
| 12 | 200 ms after step 5, the SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell. | <-- | DCI format 1_3 (only SpCell is scheduled) | - | - |
| 13 | The SS transmits a MAC PDU (containing an RLC PDU) on NR SpCell. | <-- | MAC PDU | - | - |
| 14 | Check: Does the UE transmit a Scheduling Request on PUCCH? | --> | (SR) | 1,2 | P |
| 15 | The SS sends an UL grant suitable for transmitting loop back PDU on NR SpCell. | <-- | (UL Grant) DCI format 0_1 | - | - |
| 16 | The UE transmits a MAC PDU containing the loop back PDU corresponding to step 13 on NR SpCell. | --> | MAC PDU | - | - |
| 17 | The SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell. | <-- | DCI format 1_3 (only SpCell is scheduled) | - | - |
| 18 | The SS transmits a MAC PDU containing RLC status PDU acknowledging reception of RLC PDU in step 16 on NR SpCell. | <-- | MAC PDU | - | - |
| 19 | 400 ms (sCellDeactivationTimer = 320 ms) after step 5, the SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell. | <-- | DCI format 1_3 (only SCell is scheduled) | - | - |
| 20 | The SS transmits a MAC PDU (containing an RLC PDU) on NR SCell. | <-- | MAC PDU | - | - |
| 21 | Check: Does the UE transmit a Scheduling Request on PUCCH in next 1 second? | --> | (SR) | 3 | F |
| 22 | The SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell. | <-- | DCI format 1_3 (only SpCell is scheduled) | - | - |
| 23 | The SS transmits a MAC PDU (containing an RLC PDU) on NR SpCell. | <-- | MAC PDU | - | - |
| 24 | Check: Does the UE transmit a Scheduling Request on PUCCH in next 1 second? | --> | (SR) | 3 | F |
| Note 1: Scheduling cell is NR SpCell (NR Cell 1), and reference cell is NR SCell (NR Cell 3). Therefore, the single DCI (DCI format 1_3) is counted on the NR Cell 3, and DCI size alignment procedure specified in TS 38.212 [14] clause 7.3.1.0 for NR Cell 1 is complete in step 3. |  |  |  |  |  |

7.1.1.7.4.1.3.3	Specific message contents

Table 7.1.1.7.4.1.3.3-1: RRCReconfiguration (step 1, Table 7.1.1.7.4.1.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| secondaryCellGroup | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.7.4.1.3.3-2: CellGroupConfig (Table 7.1.1.7.4.1.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| physicalCellGroupConfig | physicalCellGroupConfig | Table 7.1.1.7.4.1.3.3-5 |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated | ServingCellConfig | Table 7.1.1.7.4.1.3.3-3 |  |
| } |  |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SCellConfig { | 1 entry |  |  |
| SCellConfig[1] SEQUENCE { |  | entry 1 |  |
| sCellIndex | SCellIndex as per TS 38.508-1 [4] table 4.6.3-154 |  |  |
| sCellConfigCommon | ServingCellConfigCommon with condition No_UL and SCell_add |  |  |
| sCellConfigDedicated | ServingCellConfig | Table 7.1.1.7.4.1.3.3-6 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.7.4.1.3.3-3: ServingCellConfig (Table 7.1.1.7.4.1.3.3-2)

| Derivation Path: TS 38.508-1 [4],Table 4.6.3-167 with condition DCI 1_3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkDedicated | Table 7.1.1.7.4.1.3.3-4 |  |
| mc-DCI-SetOfCellsToAddModList-r18 SEQUENCE (SIZE (1..maxNrofSetsOfCells-r18)) OF MC-DCI-SetOfCells-r18 { | 1 entry |  |  |
| MC-DCI-SetOfCells-r18[1] SEQUENCE { |  |  |  |
| scheduledCellComboListDCI-1-3 | Not present |  | pc_coScheduledCellIndicationScheme_r18_cellInd_DCI_1_3=False AND pc_coScheduledCellIndicationScheme_r18_fdra_DCI_1_3=True |
| scheduledCellComboListDCI-1-3 SEQUENCE (SIZE (1..maxNrofCellCombos-r18)) OF ScheduledCellCombo-r18 { | 3 entries |  |  |
| ScheduledCellCombo-r18 [1] SEQUENCE (SIZE (1..maxNrofCellsInSet-r18)) OF INTEGER { |  | entry 1 |  |
| INTEGER[1] | 0 | The first entry of scheduledCellListDCI-1-3-r18 (i.e. SpCell) |  |
| } |  |  |  |
| ScheduledCellCombo-r18 [2] SEQUENCE (SIZE (1..maxNrofCellsInSet-r18)) OF INTEGER { |  | entry 2 |  |
| INTEGER[1] | 1 | The second entry of scheduledCellListDCI-1-3-r18 (i.e. SCell) |  |
| } |  |  |  |
| ScheduledCellCombo-r18 [3] SEQUENCE (SIZE (1..maxNrofCellsInSet-r18)) OF INTEGER { |  | entry 3 |  |
| INTEGER[1] | 0 | The first entry of scheduledCellListDCI-1-3-r18 (i.e. SpCell) |  |
| INTEGER[2] | 1 | The second entry of scheduledCellListDCI-1-3-r18 (i.e. SCell) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.7.4.1.3.3-4: BWP-DownlinkDedicated (Table 7.1.1.7.4.1.3.3-3)

| Derivation Path: TS 38.508-1 [4],Table 4.6.3-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkDedicated ::= SEQUENCE { |  |  |  |
| pdcch-Config CHOICE { |  |  |  |
| setup | PDCCH-Config with condition DCI_1_3 |  |  |
| } |  |  |  |
| pdsch-Config CHOICE { |  |  |  |
| setup | PDSCH-Config with condition DCI_1_3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.7.4.1.3.3-5: PhysicalCellGroupConfig (Table 7.1.1.7.4.1.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalCellGroupConfig ::= SEQUENCE { |  |  |  |
| pdsch-HARQ-ACK-Codebook | dynamic |  |  |
|  | semiStatic |  | pc_harqFeedbackType2_DCI_1_3=False AND pc_harqFeedbackType1_DCI_1_3=True |
| } |  |  |  |

Table 7.1.1.7.4.1.3.3-6: ServingCellConfig (Table 7.1.1.7.4.1.3.3-2)

| Derivation Path: TS 38.508-1 [4],Table 4.6.3-167 with condition No_UL and SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkDedicated | Table 7.1.1.7.4.1.3.3-7 |  |
| sCellDeactivationTimer | ms320 |  |  |
| } |  |  |  |

Table 7.1.1.7.4.1.3.3-7: BWP-DownlinkDedicated (Table 7.1.1.7.4.1.3.3-6)

| Derivation Path: TS 38.508-1 [4],Table 4.6.3-11 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkDedicated ::= SEQUENCE { |  |  |  |
| pdcch-Config CHOICE { |  |  |  |
| setup | PDCCH-Config | Table 7.1.1.7.4.1.3.3-8 |  |
| } |  |  |  |
| pdsch-Config CHOICE { |  |  |  |
| setup | PDSCH-Config with condition DCI_1_3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.7.4.1.3.3-8: PDCCH-Config (Table 7.1.1.7.4.1.3.3-7)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-95 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-Config ::= SEQUENCE { |  |  |  |
| controlResourceSetToAddModList | Not present |  |  |
| searchSpacesToAddModList SEQUENCE(SIZE (1..10)) OF SearchSpace { | 1 entry |  |  |
| SearchSpace[1] | SearchSpace | entry 1 Table 7.1.1.7.4.1.3.3-9 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.7.4.1.3.3-9: SearchSpace (Table 7.1.1.7.4.1.3.3-8)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | SearchSpaceId with condition DCI_MC |  |  |
| controlResourceSetId | Not present |  |  |
| monitoringSlotPeriodicityAndOffset | Not present |  |  |
| duration | Not present |  |  |
| monitoringSymbolsWithinSlot | Not present |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n4 |  | FR1 |
|  | n3 |  | FR2 |
| aggregationLevel4 | n2 |  |  |
| aggregationLevel8 | n1 |  |  |
| aggregationLevel16 | n0 |  |  |
| } |  |  |  |
| searchSpaceType | Not present |  |  |
| } |  |  |  |

Table 7.1.1.7.4.1.3.3-10: Physical layer parameters for DCI format 1_3 (steps 5, 10, 12, 17, 19 and 22 for scheduledCellComboListDCI-1-3 present configuration, Table 7.1.1.7.4.1.3.2-2)

| Derivation path: 38.508-1 [4] Table 4.3.6.1.2.4-1 with condition CellInd |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| Scheduled cells indicator | The first entry of scheduledCellComboListDCI-1-3 | “00” | Step 10,12,17,22 |
|  | The second entry of scheduledCellComboListDCI-1-3 | “01” | Step 5,19 |
| Frequency domain resource assignment | block number 1(SpCell): dependent on the Downlink TBS specified in TS 38.523-3TS Annex B.1 Note: NcellDL=1 | - | Step 10,12,17,22 |
|  | block number 1(SCell) : dependent on the Downlink TBS specified in TS 38.523-3 [3] Annex B.1 Note: NcellDL=1 | - | Step 5,19 |

Table 7.1.1.7.4.1.3.3-11: Physical layer parameters for DCI format 1_3 (steps 5, 10, 12, 17, 19 and 22 for scheduledCellComboListDCI-1-3 notpresent configuration, Table 7.1.1.7.4.1.3.2-2)

| Derivation path: TS 38.508-1 [4] Table 4.3.6.1.2.4-1 with condition FDRA and DL_CELL2 |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| Frequency domain resource assignment | block number 1(SpCell): dependent on the Downlink TBS specified in TS 38.523-3 [3] Annex B.1 block number 2(SCell) : all bits set to 1 Note: NcellDL=2 | - | Step 10,12,17,22 |
|  | block number 1(SpCell): all bits set to 1 block number 2(SCell) : dependent on the Downlink TBS specified in TS 38.523-3 [3] Annex B.1 Note: NcellDL=2 | - | Step 5,19 |

###### 7.1.1.7.4.2	Activation/Deactivation of SCells / Multi-cell PDSCH scheduling with a single DCI / Search space for single DCI is not only configured in scheduling cell / sCellDeactivationTimer / Inter-Band CA

The scope and description of the present TC is the same as test case 7.1.1.7.4.1 with the following differences:

-	CA configuration: Inter-band CA replaces Intra-band Contiguous CA

-	Cells configuration: NR Cell 10 replaces NR Cell 3

- 	NR Cell 1 and NR Cell 10 have same SCS/carrier type.

###### 7.1.1.7.4.3	Activation/Deactivation of SCells / Multi-cell PDSCH scheduling with a single DCI / Search space for single DCI is not only configured in scheduling cell / sCellDeactivationTimer / Intra-band non-Contiguous CA

The scope and description of the present TC is the same as test case 7.1.1.7.4.1 with the following differences:

-	CA configuration: Intra-band non-Contiguous CA replaces Intra-band Contiguous CA

##### 7.1.1.7.5	Activation/Deactivation of SCells / Multi-cell PUSCH scheduling with a single DCI / Search space for single DCI is not only configured in scheduling cell / sCellDeactivationTimer

###### 7.1.1.7.5.1	Activation/Deactivation of SCells / Multi-cell PUSCH scheduling with a single DCI / Search space for single DCI is not only configured in scheduling cell / sCellDeactivationTimer / Intra-band Contiguous CA

7.1.1.7.5.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with SCell configured, SCell is configured as a scheduled cell in MC-DCI-SetOfCells and with the search space for DCI to schedule multiple cells of the same searchSpaceId as the serving cell in which MC-DCI-SetOfCells containing the SCell is configured( i.e. the reference cell is SCell ) }

ensure that {

when { UE receives an SCell Activation/Deactivation MAC CE activating the Scell and sCellDeactivationTimer does not expire }

then { UE monitors PDCCH for scheduling multiple cells for the set of cells in MC-DCI-SetOfCells including the SCell(i.e. monitors DCI format 0_3 on the scheduling cell for the set of cells) }

}

(2)

with( UE in RRC_CONNECTED state with SCell activated, SCell is configured as a scheduled cell in MC-DCI-SetOfCells and with the search space for DCI to schedule multiple cells of the same searchSpaceId as the serving cell in which MC-DCI-SetOfCells containing the SCell is configured( i.e. the reference cell is SCell )  )

ensure that {

when{ UE receives a DCI format 0-3 on NR SpCell PDCCH to schedule SCell }

then {  UE restarts the sCellDeactivationTimer }

}

(3)

with ( UE in RRC_CONNECTED state with SCell activated, SCell is configured as a scheduled cell in MC-DCI-SetOfCells and with the search space for DCI to schedule multiple cells of the same searchSpaceId as the serving cell in which MC-DCI-SetOfCells containing the SCell is configured( i.e. the reference cell is SCell ))

ensure that {

when{ UE sCellDeactivationTimer expires }

then { UE deactivates the SCell and stops monitoring PDCCH for scheduling multiple cells for the set of cells in MC-DCI-SetOfCells including the SCell (i.e. stops monitoring DCI format 0_3 on the scheduling cell for the set of cells) }

}

7.1.1.7.5.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.321, clauses 5.9 and TS 38.212, clauses 7.3.1.0. Unless otherwise stated these are Rel-18 requirements.

[TS 38.321, clause 5.9]

If the MAC entity is configured with one or more SCells, the network may activate and deactivate the configured SCells. Upon configuration of an SCell, the SCell is deactivated unless the parameter sCellState is set to activated for the SCell by upper layers.

The configured SCell(s) is activated and deactivated by:

-	receiving the SCell Activation/Deactivation MAC CE described in clause 6.1.3.10;

-	receiving the Enhanced SCell Activation/Deactivation MAC CE described in clause 6.1.3.55;

-	configuring sCellDeactivationTimer timer per configured SCell (except the SCell configured with PUCCH, if any): the associated SCell is deactivated upon its expiry;

-	configuring sCellState per configured SCell: if configured, the associated SCell is activated upon SCell configuration;

-	receiving scg-State: the SCells of SCG are deactivated.

The MAC entity shall for each configured SCell:

1>	if an SCell is configured with sCellState set to activated upon SCell configuration, or an SCell Activation/Deactivation MAC CE or an Enhanced SCell Activation/Deactivation MAC CE is received activating the SCell:

…

2>	if the SCell was deactivated prior to receiving this SCell Activation/Deactivation MAC CE or this Enhanced SCell Activation/Deactivation MAC CE; or

2>	if the SCell is configured with sCellState set to activated upon SCell configuration:

3>	if firstActiveDownlinkBWP-Id is not set to dormant BWP:

4>	activate the SCell according to the timing defined in TS 38.213 [6] for MAC CE activation and according to the timing defined in TS 38.133 [11] for direct SCell activation; i.e. apply normal SCell operation including:

5>	SRS transmissions on the SCell;

5>	CSI reporting for the SCell;

5>	PDCCH monitoring on the SCell;

5>	PDCCH monitoring for the SCell;

5>	PUCCH transmissions on the SCell, if configured.

…

3>	activate the DL BWP and UL BWP indicated by firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id respectively.

2>	start or restart the sCellDeactivationTimer associated with the SCell according to the timing defined in TS 38.213 [6] for MAC CE activation and according to the timing defined in TS 38.133 [11] for direct SCell activation;

…

1>	else if an SCell Activation/Deactivation MAC CE or an Enhanced SCell Activation/Deactivation MAC CE is received deactivating the SCell; or

1>	if the sCellDeactivationTimer associated with the activated SCell expires; or

1>	if the SCG associated with the activated SCell is deactivated:

2>	deactivate the SCell according to the timing defined in TS 38.213 [6];

2>	stop the sCellDeactivationTimer associated with the SCell;

2>	stop the bwp-InactivityTimer associated with the SCell;

2>	deactivate any active BWP associated with the SCell;

2>	clear any configured downlink assignment and any configured uplink grant Type 2 associated with the SCell respectively;

2>	clear any PUSCH resource for semi-persistent CSI reporting associated with the SCell;

2>	suspend any configured uplink grant Type 1 associated with the SCell;

2>	flush all HARQ buffers associated with the SCell;

2>	cancel, if any, triggered consistent LBT failure for the SCell.

1>	if PDCCH on the activated SCell indicates an uplink grant or downlink assignment; or

1>	if PDCCH on the Serving Cell scheduling the activated SCell indicates an uplink grant or a downlink assignment for the activated SCell; or

1>	if a MAC PDU is transmitted in a configured uplink grant and LBT failure indication is not received from lower layers; or

1>	if a MAC PDU is received in a configured downlink assignment:

2>	restart the sCellDeactivationTimer associated with the SCell.

1>	if the SCell is deactivated:

2>	not transmit SRS on the SCell;

2>	not report CSI for the SCell;

2>	not transmit on UL-SCH on the SCell;

2>	not transmit on RACH on the SCell;

2>	not monitor the PDCCH on the SCell;

2>	not monitor the PDCCH for the SCell;

2>	not transmit PUCCH on the SCell;

2>	if the SCell is configured as a scheduled cell in MC-DCI-SetOfCells and with the search space for DCI to schedule multiple cells (as specified in TS 38.213 [6]) of the same searchSpaceId as the serving cell in which MC-DCI-SetOfCells containing the SCell is configured:

3>	not monitor the PDCCH for scheduling multiple cells (as specified in TS 38.213 [6]) for the set of cells in MC-DCI-SetOfCells including the SCell.

[TS 38.212, clause 7.3.1.0]

Step 3:

-	If both of the following conditions are fulfilled the size alignment procedure is complete

-	the total number of different DCI sizes configured to monitor is no more than 4 for the cell

-	the total number of different DCI sizes with C-RNTI configured to monitor is no more than 3 for the cell

[TS 38.213, clause 10.1]

A UE expects to monitor PDCCH candidates for up to 4 sizes of DCI formats that include up to 3 sizes of DCI formats with CRC scrambled by C-RNTI per serving cell. The UE counts a number of sizes for DCI formats per serving cell based on a number of configured PDCCH candidates in respective search space sets for the corresponding active DL BWP. If the UE monitors PDCCH candidates for detection of one or both of DCI format 0_3 and DCI format 1_3 for scheduling on serving cells from a set of serving cells, the serving cell for counting the size of one or both DCI format 0_3 and DCI format 1_3, respectively, is

-	the scheduling cell, if the scheduling cell is included in the set of serving cells and the UE is provided search space sets for the PDCCH candidates only on the scheduling cell

-	a serving cell from the set of serving cells, if search space sets with same searchSpaceId for one or both of DCI format 0_3 and DCI format 1_3, respectively, are provided on the serving cell and on the scheduling cell.

…

In the following, if a UE monitors PDCCH candidates on a scheduling cell for detection of DCI format 0_3 or DCI format 1_3 for scheduling on serving cells from a set of serving cells, the serving cell for counting the PDCCH candidates and a corresponding number of non-overlapping CCEs is

-	the scheduling cell, if the scheduling cell is included in the set of serving cells and the UE is provided search space sets for the PDCCH candidates only on the scheduling cell

-	a serving cell from the set of serving cells, if search space sets with same searchSpaceId for one or both of DCI format 0_3 and DCI format 1_3, respectively, are provided on the serving cell and on the scheduling cell.

7.1.1.7.5.1.3	Test description

7.1.1.7.5.1.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that DRB is configured in RLC AM mode according to Table 7.1.1.7.5.1.3.1-1 and in addition NR Cell 3 is configured as NR Active SCell.

Table 7.1.1.7.5.1.3.1-1: RLC parameters

| t-PollRetransmit | ms80 |
| --- | --- |

7.1.1.7.5.1.3.2	Test procedure sequence

Table 7.1.1.7.5.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 |
| --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -85 | -85 |

Table 7.1.1.7.5.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits an RRCReconfiguration message on NR SpCell (NR Cell 1) to configure NR SCell (NR Cell 3) and DCI format 0_3/1_3. (Note 1) | <-- | RRCReconfiguration | - | - |
| 2 | The UE transmits RRCReconfigurationComplete message. | --> | RRCReconfigurationComplete | - | - |
| 3 | The SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell. | <-- | DCI format 1_1 | - | - |
| 4 | The SS transmits Activation MAC control element to activate NR SCell on NR SpCell. | <-- | MAC PDU (SCell Activation/Deactivation MAC CE of one octet (C1=1)) | - | - |
| 5 | 200 ms after step 4, the SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell. (Note 2) | <-- | DCI format 1_1 or DCI format 1_3 (only SpCell is scheduled) | - | - |
| 6 | The SS transmits a MAC PDU (containing an RLC PDU) on NR SpCell. | <-- | MAC PDU | - | - |
| 7 | Check: Does the UE transmit a Scheduling Request on PUCCH? | --> | (SR) | 1 | P |
| 8 | The SS sends an UL grant suitable for transmitting loop back PDU on NR SpCell. The sCellDeactivationTimer is restarted in UE side. | <-- | (UL Grant) DCI format 0_3 (only SCell is scheduled) | - | - |
| 9 | The UE transmits a MAC PDU containing the loop back PDU corresponding to step 6 on NR SCell. | --> | MAC PDU | - | - |
| 10 | The SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell. (Note 2) | <-- | DCI format 1_1 or DCI format 1_3 (only SpCell is scheduled) | - | - |
| 11 | The SS transmits a MAC PDU containing RLC status PDU acknowledging reception of RLC PDU in step 9 on NR SpCell. | <-- | MAC PDU | - | - |
| 12 | 200 ms after step 5, the SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell. | <-- | DCI format 1_1 or DCI format 1_3 (only SpCell is scheduled) | - | - |
| 13 | The SS transmits a MAC PDU (containing an RLC PDU) on NR SpCell. | <-- | MAC PDU | - | - |
| 14 | Check: Does the UE transmit a Scheduling Request on PUCCH? | --> | (SR) | 1,2 | P |
| 15 | The SS sends an UL grant suitable for transmitting loop back PDU on NR SpCell. | <-- | (UL Grant) DCI format 0_3 (only SpCell is scheduled) | - | - |
| 16 | The UE transmits a MAC PDU containing the loop back PDU corresponding to step 13 on NR SpCell. | --> | MAC PDU | - | - |
| 17 | The SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell. | <-- | DCI format 1_1 or DCI format 1_3 (only SpCell is scheduled) | - | - |
| 18 | The SS transmits a MAC PDU containing RLC status PDU acknowledging reception of RLC PDU in step 16 on NR SpCell. | <-- | MAC PDU | - | - |
| 19 | 400 ms (sCellDeactivationTimer = 320 ms) after step 8, the SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell. | <-- | DCI format 1_1 | - | - |
| 20 | The SS transmits a MAC PDU (containing an RLC PDU) on NR SpCell. | <-- | MAC PDU | - | - |
| 21 | The UE transmit a Scheduling Request on PUCCH on NR SpCell. | --> | (SR) | - | - |
| 22 | The SS sends an UL grant suitable for transmitting loop back PDU on NR SpCell. | <-- | (UL Grant) DCI format 0_3 (only SpCell is scheduled) | - | - |
| 23 | Check: Does the UE transmit MAC PDU on NR SpCell? | --> | MAC PDU | 3 | F |
| 24 | The SS sends an UL grant suitable for transmitting loop back PDU on NR SpCell. | <-- | (UL Grant) DCI format 0_3 (only SCell is scheduled) | - | - |
| 25 | Check: Does the UE transmit MAC PDU on NR SCell? | --> | MAC PDU | 3 | F |
| 26 | The SS sends an UL grant suitable for transmitting loop back PDU on NR SpCell. | <-- | (UL Grant) DCI format 0_1 | - | - |
| 27 | The UE transmits a MAC PDU containing the loop back PDU corresponding to step 20 on NR SpCell. | --> | MAC PDU | - | - |
| 28 | The SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE on NR SpCell. | <-- | DCI format 1_1 | - | - |
| 29 | The SS transmits a MAC PDU containing RLC status PDU acknowledging reception of RLC PDU in step 27 on NR SpCell. | <-- | MAC PDU | - | - |
| Note 1: Scheduling cell is NR SpCell (NR Cell 1), and reference cell is NR SCell (NR Cell 3). Therefore, the single DCI (DCI format 0_3/1_3) is counted on the NR Cell 3, and DCI size alignment procedure specified in TS 38.212 [14] clause 7.3.1.0 for NR Cell 1 is complete in step3. Note 2: If the pc_multiCell_PUSCH_DCI_1_3_SameSCS_r18 = True, downlink assignment is DCI format 1_3, else the downlink assignment is DCI format 1_1. |  |  |  |  |  |

7.1.1.7.5.1.3.3	Specific message contents

Table 7.1.1.7.5.1.3.3-1: RRCReconfiguration (step 1, Table 7.1.1.7.5.1.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| secondaryCellGroup | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.7.5.1.3.3-2: CellGroupConfig (Table 7.1.1.7.5.1.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| physicalCellGroupConfig | physicalCellGroupConfig | Table 7.1.1.7.5.1.3.3-6 |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated | ServingCellConfig | Table 7.1.1.7.5.1.3.3-3 |  |
| } |  |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SCellConfig { | 1 entry |  |  |
| SCellConfig[1] SEQUENCE { |  | entry 1 |  |
| sCellIndex | SCellIndex as per TS 38.508-1 [4] table 4.6.3-154 |  |  |
| sCellConfigCommon | ServingCellConfigCommon with condition SCell_add |  |  |
| sCellConfigDedicated | ServingCellConfig | Table 7.1.1.7.5.1.3.3-7 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.7.5.1.3.3-3: ServingCellConfig (Table 7.1.1.7.5.1.3.3-2)

| Derivation Path: TS 38.508-1 [4],Table 4.6.3-167 with condition (DCI_0_3) OR (DCI_0_3 AND DCI_1_3) (NOTE 1) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkDedicated | Table 7.1.1.7.5.1.3.3-4 |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkDedicated | Table 7.1.1.7.5.1.3.3-5 |  |
| } |  |  |  |
| mc-DCI-SetOfCellsToAddModList-r18 SEQUENCE (SIZE (1..maxNrofSetsOfCells-r18)) OF MC-DCI-SetOfCells-r18 { | 1 entry |  |  |
| MC-DCI-SetOfCells-r18[1] SEQUENCE { |  |  |  |
| scheduledCellComboListDCI-1-3 | Not present |  | pc_coScheduledCellIndicationScheme_r18_cellInd_DCI_1_3=False AND pc_coScheduledCellIndicationScheme_r18_fdra_DCI_1_3=True |
| scheduledCellComboListDCI-1-3 SEQUENCE (SIZE (1..maxNrofCellCombos-r18)) OF ScheduledCellCombo-r18 { | 3 entries |  |  |
| ScheduledCellCombo-r18 [1] SEQUENCE (SIZE (1..maxNrofCellsInSet-r18)) OF INTEGER { |  | entry 1 |  |
| INTEGER[1] | 0 | The first entry of scheduledCellListDCI-1-3-r18 (i.e. SpCell) |  |
| } |  |  |  |
| ScheduledCellCombo-r18 [2] SEQUENCE (SIZE (1..maxNrofCellsInSet-r18)) OF INTEGER { |  | entry 2 |  |
| INTEGER[1] | 1 | The second entry of scheduledCellListDCI-1-3-r18 (i.e. SCell) |  |
| } |  |  |  |
| ScheduledCellCombo-r18 [3] SEQUENCE (SIZE (1..maxNrofCellsInSet-r18)) OF INTEGER { |  | entry 3 |  |
| INTEGER[1] | 0 | The first entry of scheduledCellListDCI-1-3-r18 (i.e. SpCell) |  |
| INTEGER[2] | 1 | The second entry of scheduledCellListDCI-1-3-r18 (i.e. SCell) |  |
| } |  |  |  |
| } |  |  |  |
| scheduledCellComboListDCI-0-3-r18 | Not present |  | pc_coScheduledCellIndicationScheme_r18_cellInd_DCI_0_3=False AND pc_coScheduledCellIndicationScheme_r18_fdra_DCI_0_3=True |
| scheduledCellComboListDCI-0-3-r18 SEQUENCE (SIZE (1..maxNrofCellCombos-r18)) OF ScheduledCellCombo-r18 { | 3 entries |  |  |
| ScheduledCellCombo-r18 [1] SEQUENCE (SIZE (1..maxNrofCellsInSet-r18)) OF INTEGER { |  | entry 1 |  |
| INTEGER[1] | 0 | The first entry of scheduledCellListDCI-0-3-r18 (i.e. SpCell) |  |
| } |  |  |  |
| ScheduledCellCombo-r18 [2] SEQUENCE (SIZE (1..maxNrofCellsInSet-r18)) OF INTEGER { |  | entry 2 |  |
| INTEGER[1] | 1 | The second entry of scheduledCellListDCI-0-3-r18 (i.e. SCell) |  |
| } |  |  |  |
| ScheduledCellCombo-r18 [3] SEQUENCE (SIZE (1..maxNrofCellsInSet-r18)) OF INTEGER { |  | entry 3 |  |
| INTEGER[1] | 0 | The first entry of scheduledCellListDCI-0-3-r18 (i.e. SpCell) |  |
| INTEGER[2] | 1 | The second entry of scheduledCellListDCI-0-3-r18 (i.e. SCell) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| Note 1: If the pc_multiCell_PDSCH_DCI_1_3_SameSCS_r18=True, the condition is DCI_0_3 AND DCI_1_3, else, the condition is DCI_0_3. |  |  |  |

Table 7.1.1.7.5.1.3.3-4: BWP-DownlinkDedicated (Table 7.1.1.7.5.1.3.3-3)

| Derivation Path: TS 38.508-1 [4],Table 4.6.3-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkDedicated ::= SEQUENCE { |  |  |  |
| pdcch-Config CHOICE { |  |  |  |
| setup | PDCCH-Config with condition DCI_0_3 |  |  |
|  | PDCCH-Config with condition DCI_0_3 and DCI_1_3 |  | pc_multiCell_PDSCH_DCI_1_3_SameSCS_r18 |
| } |  |  |  |
| pdsch-Config CHOICE { |  |  | pc_multiCell_PDSCH_DCI_1_3_SameSCS_r18 |
| setup | PDSCH-Config with condition DCI_1_3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.7.5.1.3.3-5: BWP-UplinkDedicated (Table 7.1.1.7.5.1.3.3-3)

| Derivation Path: TS 38.508-1 [4],Table 4.6.3-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicated ::= SEQUENCE { |  |  |  |
| pusch-Config CHOICE { |  |  |  |
| setup | PUSCH-Config with condition DCI_0_3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.7.5.1.3.3-6: PhysicalCellGroupConfig (Table 7.1.1.7.5.1.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalCellGroupConfig ::= SEQUENCE { |  |  |  |
| pdsch-HARQ-ACK-Codebook | dynamic |  | pc_harqFeedbackType1_DCI_1_3=False AND pc_harqFeedbackType2_DCI_1_3=True |
|  | semiStatic |  |  |
| } |  |  |  |

Table 7.1.1.7.5.1.3.3-7: ServingCellConfig (Table 7.1.1.7.5.1.3.3-2)

| Derivation Path: TS 38.508-1 [4],Table 4.6.3-167 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkDedicated | Table 7.1.1.7.5.1.3.3-8 |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkDedicated | Table 7.1.1.7.5.1.3.3-9 |  |
| } |  |  |  |
| sCellDeactivationTimer | ms320 |  |  |
| } |  |  |  |

Table 7.1.1.7.5.1.3.3-8: BWP-DownlinkDedicated (Table 7.1.1.7.5.1.3.3-7)

| Derivation Path: TS 38.508-1 [4],Table 4.6.3-11 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkDedicated ::= SEQUENCE { |  |  |  |
| pdcch-Config CHOICE { |  |  |  |
| setup | PDCCH-Config | Table 7.1.1.7.5.1.3.3-10 |  |
| } |  |  |  |
| pdsch-Config CHOICE { |  |  |  |
| setup | PDSCH-Config with condition DCI_1_3 |  | pc_multiCell_PDSCH_DCI_1_3_SameSCS_r18 |
|  | PDSCH-Config |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.7.5.1.3.3-9: BWP-UplinkDedicated (Table 7.1.1.7.5.1.3.3-7)

| Derivation Path: TS 38.508-1 [4],Table 4.6.3-15 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicated ::= SEQUENCE { |  |  |  |
| pusch-Config CHOICE { |  |  |  |
| setup | PUSCH-Config with condition DCI_0_3 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.7.5.1.3.3-10: PDCCH-Config (Table 7.1.1.7.5.1.3.3-8)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-95 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-Config ::= SEQUENCE { |  |  |  |
| controlResourceSetToAddModList | Not present |  |  |
| searchSpacesToAddModList SEQUENCE(SIZE (1..10)) OF SearchSpace { | 1 entry |  |  |
| SearchSpace[1] | SearchSpace | entry 1 Table 7.1.1.7.5.1.3.3-11 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.7.5.1.3.3-11: SearchSpace (Table 7.1.1.7.5.1.3.3-10)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | SearchSpaceId with condition DCI_MC |  |  |
| controlResourceSetId | Not present |  |  |
| monitoringSlotPeriodicityAndOffset | Not present |  |  |
| duration | Not present |  |  |
| monitoringSymbolsWithinSlot | Not present |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n4 |  | FR1 |
|  | n3 |  | FR2 |
| aggregationLevel4 | n2 |  |  |
| aggregationLevel8 | n1 |  |  |
| aggregationLevel16 | n0 |  |  |
| } |  |  |  |
| searchSpaceType | Not present |  |  |
| } |  |  |  |

Table 7.1.1.7.5.1.3.3-10: Physical layer parameters for DCI format 1_3 (steps 5, 10, 12 and 17 for scheduledCellComboListDCI-1-3 present configuration, Table 7.1.1.7.5.1.3.2-2)

| Derivation path: TS 38.508-1 [4] Table 4.3.6.1.2.4-1 with condition CellInd |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| Scheduled cells indicator | The first entry of scheduledCellComboListDCI-1-3 | “00” |  |
| Frequency domain resource assignment | block number 1(SpCell): dependent on the Downlink TBS in TS 38.523-3 [3] Annex B.1 Note: NcellDL=1 | - |  |

Table 7.1.1.7.5.1.3.3-11: Physical layer parameters for DCI format 1_3 (steps 5, 10, 12 and 17 for scheduledCellComboListDCI-1-3 not present configuration, Table 7.1.1.7.5.1.3.2-2)

| Derivation path: TS 38.508-1 [4] Table 4.3.6.1.2.4-1 with condition FDRA and DL_CELL2 |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| Frequency domain resource assignment | block number 1(SpCell): dependent on the Downlink TBS specified in TS 38.523-3 [3] Annex B.1 block number 2(SCell) : all bits set to 1 Note: NcellDL=2 | - |  |

Table 7.1.1.7.5.1.3.3-12: Physical layer parameters for DCI format 0_3 (steps 8, 15, 22 and 24 for scheduledCellComboListDCI-0-3 present configuration, Table 7.1.1.7.5.1.3.2-2)

| Derivation path: TS 38.508-1 [4] Table 4.3.6.1.2.4-1 with condition CellInd |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| Scheduled cells indicator | The first entry of scheduledCellComboListDCI-1-3 | “00” | Step 15,22 |
|  | The second entry of scheduledCellComboListDCI-1-3 | “01” | Step 8,24 |
| Frequency domain resource assignment | block number 1(SpCell): dependent on the Downlink TBS specified in TS 38.523-3 [3] Annex B.1 Note: NcellDL=1 | - | Step 15,22 |
|  | block number 1(SCell) : dependent on the Downlink TBS specified in TS 38.523-3 [3] Annex B.1 Note: NcellDL=1 | - | Step 8,24 |

Table 7.1.1.7.5.1.3.3-13: Physical layer parameters for DCI format 0_3 (steps 8, 15, 22 and 24 for scheduledCellComboListDCI-0-3 notpresent configuration, Table 7.1.1.7.5.1.3.2-2)

| Derivation path: TS 38.508-1 [4] Table 4.3.6.1.2.4-1 with condition FDRA and DL_CELL2 |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| Frequency domain resource assignment | block number 1(SpCell): dependent on the Downlink TBS specified in TS 38.523-3 [3] Annex B.1 block number 2(SCell) : all bits set to 1 Note: NcellDL=2 | - | Step 15,22 |
|  | block number 1(SpCell): all bits set to 1 block number 2(SCell) : dependent on the Downlink TBS specified in TS 38.523-3 [3] Annex B.1 Note: NcellDL=2 | - | Step 8,24 |

###### 7.1.1.7.5.2	Activation/Deactivation of SCells / Multi-cell PUSCH scheduling with a single DCI / Search space for single DCI is not only configured in scheduling cell / sCellDeactivationTimer / Inter-Band CA

The scope and description of the present TC is the same as test case 7.1.1.7.5.1 with the following differences:

-	CA configuration: Inter-band CA replaces Intra-band Contiguous CA

-	Cells configuration: NR Cell 10 replaces NR Cell 3

- 	NR Cell 1 and NR Cell 10 have same SCS/carrier type.

###### 7.1.1.7.5.3	Activation/Deactivation of SCells / Multi-cell PUSCH scheduling with a single DCI / Search space for single DCI is not only configured in scheduling cell / sCellDeactivationTimer / Intra-band non-Contiguous CA

The scope and description of the present TC is the same as test case 7.1.1.7.5.1 with the following differences:

-	CA configuration: Intra-band non-Contiguous CA replaces Intra-band Contiguous CA

#### 7.1.1.8	Bandwidth Part (BWP) operation

##### 7.1.1.8.1	Bandwidth Part (BWP) operation UL/DL

7.1.1.8.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives BandwidthPart-Config IE included in RRC Message received on SpCell (i.e. PSCell in case of EN-DC or PCell in case of SA) }

then { UE starts normal MAC operation in the FirstActive UL and DL Bandwidth part }

}

(2)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives a DL DCI format 1_1 assigning a BWP different than the previously configured BWP }

then { UE starts normal MAC operation in the received new BWP }

}

(3)

with { UE in RRC_CONNECTED }

ensure that {

when { UE receives a UL DCI format 0_1 assigning a BWP different than the previously configured BWP }

then { UE starts normal MAC operation in the received new BWP }

}

(4)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE determines that a RACH Procedure is triggered in SpCell (i.e. PSCell in case of EN-DC or PCell in case of SA) and PRACH occasions are not configured }

then { UE initiates the PRACH procedure in the initial BWP }

}

(5)

with { UE in RRC_Connected State with defaultDownlinkBWP configured }

ensure that {

when { UE bwp-InactivityTimer expires }

then { UE performs BWP switching to a BWP indicated by the defaultDownlinkBWP }

}

(6)

with { UE in RRC_Connected State with defaultDownlinkBWP configured and Active BWP is different than defaultDownlinkBWP and bwp-InactivityTimer is running }

ensure that {

when { UE receives UL assignment or DL grant addressed to its C-RNTI }

then { UE restarts the bwp-InactivityTimer }

}

7.1.1.8.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.211 clause 4.4.5, TS 38.212 clause 7.3.1.1.2 and 7.3.1.2.2, TS 38.321 clause 5.15 and TS 38.331 clause 5.3.5.3. Unless otherwise stated these are Rel-15 requirements.

[TS 38.211, clause 4.4.5]

A bandwidth part is a subset of contiguous common resource blocks defined in subclause 4.4.4.3 for a given numerology  in bandwidth part  on a given carrier. The starting position  and the number of resource blocks  in a bandwidth part shall fulfil  and , respectively. Configuration of a bandwidth part is described in clause 12 of [5, TS 38.213].

A UE can be configured with up to four bandwidth parts in the downlink with a single downlink bandwidth part being active at a given time. The UE is not expected to receive PDSCH, PDCCH, or CSI-RS (except for RRM) outside an active bandwidth part.

A UE can be configured with up to four bandwidth parts in the uplink with a single uplink bandwidth part being active at a given time. If a UE is configured with a supplementary uplink, the UE can in addition be configured with up to four bandwidth parts in the supplementary uplink with a single supplementary uplink bandwidth part being active at a given time. The UE shall not transmit PUSCH or PUCCH outside an active bandwidth part. For an active cell, the UE shall not transmit SRS outside an active bandwidth part.

Unless otherwise noted, the description in this specification applies to each of the bandwidth parts. When there is no risk of confusion, the index  may be dropped from , , , and .

[TS 38.212, clause 7.3.1.1.2]

DCI format 0_1 is used for the scheduling of PUSCH in one cell.

The following information is transmitted by means of the DCI format 0_1 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats – 1 bit

-	The value of this bit field is always set to 0, indicating an UL DCI format

-	Carrier indicator – 0 or 3 bits, as defined in Subclause 10.1 of [5, TS 38.213].

-	UL/SUL indicator – 0 bit for UEs not configured with SUL in the cell or UEs configured with SUL in the cell but only PUCCH carrier in the cell is configured for PUSCH transmission; 1 bit for UEs configured with SUL in the cell as defined in Table 7.3.1.1.1-1.

-	Bandwidth part indicator – 0, 1 or 2 bits as determined by the number of UL BWPs  configured by higher layers, excluding the initial UL bandwidth part. The bitwidth for this field is determined as bits, where

-	 if , in which case the bandwidth part indicator is equivalent to the ascending order of the higher layer parameter BWP-Id;

-	otherwise , in which case the bandwidth part indicator is defined in Table 7.3.1.1.2-1;

If a UE does not support active BWP change via DCI, the UE ignores this bit field.

[TS 38.212, clause 7.3.1.2.2]

DCI format 1_1 is used for the scheduling of PDSCH in one cell.

The following information is transmitted by means of the DCI format 1_1 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats – 1 bits

-	The value of this bit field is always set to 1, indicating a DL DCI format

-	Carrier indicator – 0 or 3 bits as defined in Subclause 10.1 of [5, TS 38.213].

-	Bandwidth part indicator – 0, 1 or 2 bits as determined by the number of DL BWPs  configured by higher layers, excluding the initial DL bandwidth part. The bitwidth for this field is determined as bits, where

-	 if , in which case the bandwidth part indicator is equivalent to the higher layer parameter BWP-Id;

-	otherwise , in which case the bandwidth part indicator is defined in Table 7.3.1.1.2-1;

If a UE does not support active BWP change via DCI, the UE ignores this bit field.

[TS 38.321, clause 5.15]

In addition to clause 12 of TS 38.213 [6], this subclause specifies requirements on BWP operation.

A Serving Cell may be configured with one or multiple BWPs, and the maximum number of BWP per Serving Cell is specified in TS 38.213 [6].

The BWP switching for a Serving Cell is used to activate an inactive BWP and deactivate an active BWP at a time. The BWP switching is controlled by the PDCCH indicating a downlink assignment or an uplink grant, by the bwp-InactivityTimer, by RRC signalling, or by the MAC entity itself upon initiation of Random Access procedure. Upon RRC (re-)configuration of firstActiveDownlinkBWP-Id and/or firstActiveUplinkBWP-Id for SpCell or activation of an SCell, the DL BWP and/or UL BWP indicated by firstActiveDownlinkBWP-Id and/or firstActiveUplinkBWP-Id respectively (as specified in TS 38.331 [5]) is active without receiving PDCCH indicating a downlink assignment or an uplink grant. The active BWP for a Serving Cell is indicated by either RRC or PDCCH (as specified in TS 38.213 [6]). For unpaired spectrum, a DL BWP is paired with a UL BWP, and BWP switching is common for both UL and DL.

For each activated Serving Cell configured with a BWP, the MAC entity shall:

1>	if a BWP is activated:

2>	transmit on UL-SCH on the BWP;

2>	transmit on RACH on the BWP, if PRACH occasions are configured;

2>	monitor the PDCCH on the BWP;

2>	transmit PUCCH on the BWP, if configured;

2>	report CSI for the BWP;

2>	transmit SRS on the BWP, if configured;

2>	receive DL-SCH on the BWP;

2>	(re-)initialize any suspended configured uplink grants of configured grant Type 1 on the active BWP according to the stored configuration, if any, and to start in the symbol according to rules in subclause 5.8.2.

1>	if a BWP is deactivated:

2>	not transmit on UL-SCH on the BWP;

2>	not transmit on RACH on the BWP;

2>	not monitor the PDCCH on the BWP;

2>	not transmit PUCCH on the BWP;

2>	not report CSI for the BWP;

2>	not transmit SRS on the BWP;

2>	not receive DL-SCH on the BWP;

2>	clear any configured downlink assignment and configured uplink grant of configured grant Type 2 on the BWP;

2>	suspend any configured uplink grant of configured grant Type 1 on the inactive BWP.

Upon initiation of the Random Access procedure on a Serving Cell, after the selection of carrier for performing Random Access procedure as specified in subclause 5.1.1, the MAC entity shall for the selected carrier of this Serving Cell:

1>	if PRACH occasions are not configured for the active UL BWP:

2>	switch the active UL BWP to BWP indicated by initialUplinkBWP;

2>	if the Serving Cell is a SpCell:

3>	switch the active DL BWP to BWP indicated by initialDownlinkBWP.

1>	else:

2>	if the Serving Cell is a SpCell:

3>	if the active DL BWP does not have the same bwp-Id as the active UL BWP:

4>	switch the active DL BWP to the DL BWP with the same bwp-Id as the active UL BWP.

1>	stop the bwp-InactivityTimer associated with the active DL BWP of this Serving Cell, if running.

1>	if the Serving Cell is SCell:

2>	stop the bwp-InactivityTimer associated with the active DL BWP of SpCell, if running.

1>	perform the Random Access procedure on the active DL BWP of SpCell and active UL BWP of this Serving Cell.

If the MAC entity receives a PDCCH for BWP switching of a Serving Cell, the MAC entity shall:

1>	if there is no ongoing Random Access procedure associated with this Serving Cell; or

1>	if the ongoing Random Access procedure associated with this Serving Cell is successfully completed upon reception of this PDCCH addressed to C-RNTI (as specified in subclauses 5.1.4 and 5.1.5):

2>	perform BWP switching to a BWP indicated by the PDCCH.

If the MAC entity receives a PDCCH for BWP switching for a Serving Cell while a Random Access procedure associated with that Serving Cell is ongoing in the MAC entity, it is up to UE implementation whether to switch BWP or ignore the PDCCH for BWP switching, except for the PDCCH reception for BWP switching addressed to the C-RNTI for successful Random Access procedure completion (as specified in subclauses 5.1.4 and 5.1.5) in which case the UE shall perform BWP switching to a BWP indicated by the PDCCH. Upon reception of the PDCCH for BWP switching other than successful contention resolution, if the MAC entity decides to perform BWP switching, the MAC entity shall stop the ongoing Random Access procedure and initiate a Random Access procedure after performing the BWP switching; if the MAC decides to ignore the PDCCH for BWP switching, the MAC entity shall continue with the ongoing Random Access procedure on the Serving Cell.

Upon reception of RRC (re-)configuration for BWP switching for a Serving Cell while a Random Access procedure associated with that Serving Cell is ongoing in the MAC entity, the MAC entity shall stop the ongoing Random Access procedure and initiate a Random Access procedure after performing the BWP switching.

The MAC entity shall for each activated Serving Cell configured with bwp-InactivityTimer:

1>	if the defaultDownlinkBWP-Id is configured, and the active DL BWP is not the BWP indicated by the defaultDownlinkBWP-Id; or

1>	if the defaultDownlinkBWP-Id is not configured, and the active DL BWP is not the initialDownlinkBWP:

2>	if a PDCCH addressed to C-RNTI or CS-RNTI indicating downlink assignment or uplink grant is received on the active BWP; or

2>	if a PDCCH addressed to C-RNTI or CS-RNTI indicating downlink assignment or uplink grant is received for the active BWP; or

2>	if a MAC PDU is transmitted in a configured uplink grant or received in a configured downlink assignment:

3>	if there is no ongoing random access procedure associated with this Serving Cell; or

3>	if the ongoing Random Access procedure associated with this Serving Cell is successfully completed upon reception of this PDCCH addressed to C-RNTI (as specified in subclauses 5.1.4 and 5.1.5):

4>	start or restart the bwp-InactivityTimer associated with the active DL BWP.

2>	if the bwp-InactivityTimer associated with the active DL BWP expires:

3>	if the defaultDownlinkBWP-Id is configured:

4>	perform BWP switching to a BWP indicated by the defaultDownlinkBWP-Id.

3>	else:

4>	perform BWP switching to the initialDownlinkBWP.

NOTE:	If a Random Access procedure is initiated on an SCell, both this SCell and the SpCell are associated with this Random Access procedure.

1>	if a PDCCH for BWP switching is received, and the MAC entity switches the active DL BWP:

2>	if the defaultDownlinkBWP-Id is configured, and the MAC entity switches to the DL BWP which is not indicated by the defaultDownlinkBWP-Id; or

2>	if the defaultDownlinkBWP-Id is not configured, and the MAC entity switches to the DL BWP which is not the initialDownlinkBWP:

3>	start or restart the bwp-InactivityTimer associated with the active DL BWP.

[TS 38.331, clause 5.2.1]

System Information (SI) is divided into the MIB and a number of SIBs where:

-	...

-	For a UE in RRC_CONNECTED, the network can provide system information through dedicated signalling using the RRCReconfiguration message, e.g. if the UE has an active BWP with no common search space configured to monitor system information or paging.

-	For PSCell and SCells, the network provides the required SI by dedicated signalling, i.e. within an RRCReconfiguration message. Nevertheless, the UE shall acquire MIB of the PSCell to get SFN timing of the SCG (which may be different from MCG). Upon change of relevant SI for SCell, RAN releases and adds the concerned SCell. For PSCell, SI can only be changed with Reconfiguration with Sync.

NOTE:	The physical layer imposes a limit to the maximum size a SIB can take. The maximum SIB1 or SI message size is 2976 bits.

[TS 38.331, clause 5.3.5.3]

The UE shall perform the following actions upon reception of the RRCReconfiguration:

...

1>	if the UE is configured with E-UTRA nr-SecondaryCellGroupConfig (MCG is E-UTRA):

2>	if RRCReconfiguration was received via SRB1:

3>	submit the RRCReconfigurationComplete via the EUTRA MCG embedded in E-UTRA RRC message RRCConnectionReconfigurationComplete as specified in TS 36.331 [10];

3>	if reconfigurationWithSync was included in spCellConfig of an SCG:

4>	initiate the random access procedure on the SpCell, as specified in TS 38.321 [3];

...

NOTE:	For EN-DC, in the case RRCReconfiguration is received via SRB1, the random access is triggered by RRC layer itself as there is not necessarily other UL transmission. In the case RRCReconfiguration is received via SRB3, the random access is triggered by the MAC layer due to arrival of RRCReconfigurationComplete.

7.1.1.8.1.3	Test description

7.1.1.8.1.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0.

7.1.1.8.1.3.2	Test procedure sequence

Table 7.1.1.8.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | The SS transmits RRCReconfiguration to configure the dedicated BWPs incl. the FirstActive BWP. (Note 1) (Note 4). | <-- | RRCReconfiguration | - | - |
| - | EXCEPTION: Steps 0Aa1 to 0Ab2 describe behaviour which depends on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value. | - | - | - | - |
| 0Aa1 | IF Connectivity is EN-DC or NGEN-DC, the UE sends RRCReconfigurationComplete (Note 2). | --> | RRCReconfigurationComplete | - | - |
| 0Ab1 | IF Connectivity is NR, the SS allocates (transmitted in FirstActiveDownlinkBWP) an UL Grant with DCI format 0_1 indicating FirstActiveUplinkBWP (BWP#1). | <-- | UL Grant | - | - |
| 0Ab2 | Check: Does the UE send RRCReconfigurationComplete in the FirstActive BWP configured? Note 3 (Note 5) | --> | (RRCReconfigurationComplete) | 1 | P |
| 1 | The SS transmits a valid MAC PDU containing RLC PDU in the configured FirstActive Downlink BWP configured. | <-- | MAC PDU | - | - |
| 2 | After 100ms from step 1, the SS allocates (transmitted in FirstActiveDownlinkBWP) an UL Grant. | <-- | UL Grant | - | - |
| 3 | Check: Does the UE transmit a MAC PDU including one RLC SDU in the FirstActive BWP configured? (Note 5) | --> | MAC PDU | 1 | P |
| 3A | SS transmits a MAC PDU containing an RLC STATUS PDU acknowledging the reception of the AMD PDU in step 3. | <-- | MAC PDU (RLC STATUS PDU) | - | - |
| 4 | Void | - | - | - | - |
|  | EXCEPTION: Step 5 describes behaviour that depends on the UE capability. |  |  |  |  |
| 5a | IF (pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto4_FR2 and band under test is FR2) THEN The SS indicates on PDCCH (transmitted in Downlink BWP#1) DL DCI format 1_1 with new BWP Id BWP#2 and transmits a MAC PDU containing RLC PDU on the newly configured BWP (BWP#2). | <-- | MAC PDU | - | - |
| 5b | IF pc_bwp_sameNumerology_upto4_FR1_FDD and band under test is FR1 FDD THEN The SS indicates on PDCCH (transmitted in Downlink BWP#1) DL DCI format 1_1 with new BWP Id BWP#2 and transmits a MAC PDU containing RLC PDU on the newly configured BWP (BWP#2). | <-- | MAC PDU | - | - |
| 5c | IF (pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto2_FR2 and band under test is FR2) THEN The SS indicates on PDCCH (transmitted in Downlink BWP#1) DL DCI format 1_1 with new BWP Id BWP#0 and transmits a MAC PDU containing RLC PDU on the newly configured BWP (BWP#0). | <-- | MAC PDU | - | - |
| 5d | IF pc_bwp_sameNumerology_upto2_FR1_FDD and band under test is FR1 FDD THEN The SS indicates on PDCCH (transmitted in Downlink BWP#1) DL DCI format 1_1 with new BWP Id BWP#0 and transmits a MAC PDU containing RLC PDU on the newly configured BWP (BWP#0). | <-- | MAC PDU | - | - |
|  | EXCEPTION: Step 6 describes behaviour that depends on the UE capability. |  |  |  |  |
| 6a | IF (pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto4_FR2 and band under test is FR2) THEN After 100ms from step 5, the SS allocates (transmitted in Downlink BWP#2) an UL Grant (with DCI indicating BWP#2), sufficient for loopback of the RLC SDU from step 5 in a Slot. (Note 3) | <-- | UL Grant | - | - |
| 6b | IF pc_bwp_sameNumerology_upto4_FR1_FDD and band under test is FR1 FDD THEN After 100ms from step 5, the SS allocates (transmitted in Downlink BWP#2) an UL Grant (with DCI indicating BWP#1), sufficient for loopback of the RLC SDU from step 5 in a Slot. (Note 3) | <-- | UL Grant | - | - |
| 6c | IF (pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto2_FR2 and band under test is FR2) THEN After 100ms from step 5, the SS allocates (transmitted in Downlink BWP#0) an UL Grant (with DCI indicating BWP#0), sufficient for loopback of the RLC SDU from step 5 in a Slot. (Note 3) | <-- | UL Grant | - | - |
| 6d | IF pc_bwp_sameNumerology_upto2_FR1_FDD and band under test is FR1 FDD THEN After 100ms from step 5, the SS allocates (transmitted in Downlink BWP#0) an UL Grant (with DCI indicating BWP#1), sufficient for loopback of the RLC SDU from step 5 in a Slot. (Note 3) | <-- | UL Grant | - | - |
|  | EXCEPTION: Step 7 describes behaviour that depends on the UE capability. |  |  |  |  |
| 7a | IF (pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto4_FR2 and band under test is FR2) THEN Check: Does the UE transmit a MAC PDU including one RLC SDU in the configured BWP (i.e. Uplink BWP#2)? (Note 5) | --> | MAC PDU | 2 | P |
| 7b | IF pc_bwp_sameNumerology_upto4_FR1_FDD and band under test is FR1 FDD THEN Check: Does the UE transmit a MAC PDU including one RLC SDU in the configured BWP (i.e. Uplink BWP#1)? (Note 5) | --> | MAC PDU | 2 | P |
| 7c | IF (pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto2_FR2 and band under test is FR2) THEN Check: Does the UE transmit a MAC PDU including one RLC SDU in the configured BWP (i.e. Uplink BWP#0)? (Note 5) | --> | MAC PDU | 2 | P |
| 7d | IF pc_bwp_sameNumerology_upto2_FR1_FDD and band under test is FR1 FDD THEN Check: Does the UE transmit a MAC PDU including one RLC SDU in the configured BWP (i.e. Uplink BWP#1)? (Note 5) | --> | MAC PDU | 2 | P |
| 7A | SS transmits a MAC PDU containing an RLC STATUS PDU acknowledging the reception of the AMD PDU in step 7. | <-- | MAC PDU (RLC STATUS PDU) | - |  |
| 8 | Void | - | - | - | - |
|  | EXCEPTION: Step 9 describes behaviour that depends on the UE capability. |  |  |  |  |
| 9a | IF (pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto4_FR2 and band under test is FR2) THEN The SS transmits a valid MAC PDU containing RLC PDU in the configured BWP (i.e. Downlink BWP#2). | <-- | MAC PDU | - | - |
| 9b | IF pc_bwp_sameNumerology_upto4_FR1_FDD and band under test is FR1 FDD THEN The SS transmits a valid MAC PDU containing RLC PDU in the configured BWP (i.e. Downlink BWP#2). | <-- | MAC PDU | - | - |
| 9c | IF (pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto2_FR2 and band under test is FR2) THEN The SS transmits a valid MAC PDU containing RLC PDU in the configured BWP (i.e. Downlink BWP#0). | <-- | MAC PDU | - | - |
| 9d | IF pc_bwp_sameNumerology_upto2_FR1_FDD and band under test is FR1 FDD THEN The SS transmits a valid MAC PDU containing RLC PDU in the configured BWP (i.e. Downlink BWP#0). | <-- | MAC PDU | - | - |
|  | EXCEPTION: Step 10 describe behaviour that depends on the UE capability. |  |  |  |  |
| 10a | IF pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto4_FR2 and band under test is FR2) THEN after 100ms from step 9 the SS indicates on PDCCH (transmitted in Downlink BWP#2) an UL DCI format 0_1 with new BWP Id BWP#3 and allocates an UL Grant, sufficient for loopback of the RLC SDU from step 9. | <-- | UL Grant | - | - |
| 10b | IF pc_bwp_sameNumerology_upto4_FR1_FDD and band under test is FR1 FDD THEN After 100ms from step 9 the SS indicates on PDCCH (transmitted in Downlink BWP#2) an UL DCI format 0_1 with new BWP Id BWP#3 and allocates an UL Grant, sufficient for loopback of the RLC SDU from step 9. | <-- | UL Grant | - | - |
| 10c | IF (pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto2_FR2 and band under test is FR2) THEN After 100ms from step 9 the SS indicates on PDCCH (transmitted in Downlink BWP#0 an UL DCI format 0_1 with new BWP Id BWP#1 and allocates an UL Grant, sufficient for loopback of the RLC SDU from step 9. | <-- | UL Grant | - | - |
| 10d | IF pc_bwp_sameNumerology_upto2_FR1_FDD and band under test is FR1 FDD THEN After 100ms from step 9 the SS indicates on PDCCH (transmitted in Downlink BWP#0 an UL DCI format 0_1 with BWP#0 and allocates an UL Grant, sufficient for loopback of the RLC SDU from step 9. | <-- | UL Grant | - | - |
|  | EXCEPTION: Step 11 describes behaviour that depends on the UE capability. |  |  |  |  |
| 11a | IF (pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto4_FR2 and band under test is FR2) THEN Check: Does UE transmit a MAC PDU including one RLC SDU in the configured BWP (i.e. Uplink BWP#3)? (Note 5) | --> | MAC PDU | 3 | P |
| 11b | IF pc_bwp_sameNumerology_upto4_FR1_FDD and band under test is FR1 FDD THEN Check: Does UE transmit a MAC PDU including one RLC SDU in the configured BWP (i.e. Uplink BWP#3)? (Note 5) | --> | MAC PDU | 3 | P |
| 11c | IF (pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto2_FR2 and band under test is FR2) THEN Check: Does UE transmit a MAC PDU including one RLC SDU in the configured BWP (i.e. Uplink BWP#1)? (Note 5) | --> | MAC PDU | 3 | P |
| 11d | IF pc_bwp_sameNumerology_upto2_FR1_FDD and band under test is FR1 FDD THEN Check: Does UE transmit a MAC PDU including one RLC SDU in the configured BWP (i.e. Uplink BWP#0)? (Note 5) | --> | MAC PDU | 3 | P |
| 11AA | SS transmits a MAC PDU containing an RLC STATUS PDU acknowledging the reception of the AMD PDUs in step 11. | <-- | MAC PDU (RLC STATUS PDU) | - | - |
|  | EXCEPTION: Step 11AAB describes behaviour that depends on the UE capability. Note: for FDD and pc_bwp_sameNumerology_upto2_FR1_FDD switch DL and UL BWP from BWP#0 to (BWP#1). |  |  |  |  |
| 11AABa1 | IF pc_bwp_sameNumerology_upto2_FR1_FDD and band under test is FR1 FDD THEN The SS indicates on PDCCH (transmitted in Downlink BWP#0) DL DCI format 1_1 with new BWP Id BWP#1 and transmits a MAC PDU containing RLC PDU on the newly configured BWP (BWP#1). | --> | MAC PDU | - | - |
| 11AABa2 | After 100ms from step 11AABa1 the SS indicates on PDCCH (transmitted in Downlink BWP#1 an UL DCI format 0_1 with new BWP Id BWP#1 and allocates an UL Grant, sufficient for loopback of the RLC SDU from step 11AABa1. | <-- | UL Grant | - | - |
| 11AABa3 | UE transmits a MAC PDU including one RLC SDU in the configured BWP (i.e. Uplink (BWP#1) | --> | MAC PDU | - | - |
| 11AABa4 | SS transmits a MAC PDU containing an RLC STATUS PDU acknowledging the reception of the AMD PDUs in step 11AABa3. | <-- | MAC PDU (RLC STATUS PDU) | - | - |
|  | EXCEPTION: Step 11A describes behaviour that depends on the UE capability. |  |  |  |  |
| 11Aa | IF (pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto4_FR2 and band under test is FR2) THEN the SS transmits a valid MAC PDU containing RLC PDU in the configured BWP (BWP#3) | <-- | MAC PDU | - | - |
| 11Ab | IF pc_bwp_sameNumerology upto4_FR1_FDD and band under test is FR1 FDD THEN the SS indicates on PDCCH (transmitted in Downlink BWP#2) DL DCI format 1_1 with new BWP Id BWP#3 and transmits a MAC PDU containing RLC PDU on the newly configured BWP (BWP#3). | <-- | MAC PDU | - | - |
| 11Ac | IF (pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto2_FR2 and band under test is FR2) THEN the SS transmits a valid MAC PDU containing RLC PDU in the configured BWP (BWP#1). | <-- | MAC PDU | - | - |
| 11Ad | IF pc_bwp_sameNumerology_upto2_FR1_FDD and band under test is FR1 FDD THEN the SS transmits a valid MAC PDU containing RLC PDU in the configured BWP (BWP#1). | <-- | MAC PDU | - | - |
|  | EXCEPTION: Step 12 describes behaviour that depends on the UE capability. |  |  |  |  |
| 12a | IF (pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto4_FR2 and band under test is FR2) THEN after 100ms from step 11A the SS indicates PDCCH order on CSS for contention-based random access (transmitted in Downlink BWP (BWP#3). | <-- | PDCCH Order (ra-PreambleIndex = '000000'B) | - | - |
| 12b | IF pc_bwp_sameNumerology_upto4_FR1_FDD and band under test is FR1 FDD THEN after 100ms from step 11A the SS indicates PDCCH order on CSS for contention-based random access (transmitted in Downlink BWP (BWP#3). | <-- | PDCCH Order (ra-PreambleIndex = '000000'B) | - | - |
| 12c | IF (pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto2_FR2 and band under test is FR2) THEN after 100ms from step 11A the SS indicates PDCCH order on CSS for contention-based random access (transmitted in Downlink BWP (BWP#1). | <-- | PDCCH Order (ra-PreambleIndex = '000000'B) | - | - |
| 12d | IF pc_bwp_sameNumerology_upto2_FR1_FDD and band under test is FR1 FDD THEN after 100ms from step 11A the SS indicates PDCCH order on CSS for contention-based random access (transmitted in Downlink BWP (BWP#1). | <-- | PDCCH Order (ra-PreambleIndex = '000000'B) | - | - |
| 13 | Check: Does the UE send PRACH Preamble in the initial BWP (UL BWP#0)? | --> | PRACH Preamble | 4 | P |
| 13A | The SS transmits (in Downlink BWP#0) a MAC PDU addressed to UE RA-RNTI, containing RAR with matching RAPID in MAC sub header. | <-- | Random Access Response | - | - |
| 13B | The UE sends (in UL BWP#0) a msg3 in the grant associated to the received Random Access Response. | --> | msg3 (C-RNTI MAC CONTROL ELEMENT) | - | - |
| 13C | SS schedules (in Downlink BWP#0) PDCCH transmission for UE C-RNTI and allocates UL grant sufficient for the UE to loop back the data received at step 11A. | <-- | Contention Resolution | - | - |
| 13D | Check: Does the UE transmit a MAC PDU including one RLC SDU in the initial BWP (i.e. Uplink BWP#0)? (Note 5) | --> | MAC PDU | 4 | P |
| 13E | SS transmits a MAC PDU containing an RLC STATUS PDU acknowledging the reception of the AMD PDU in step 13D. | <-- | MAC PDU (RLC STATUS PDU) | - |  |
| 14-15 | Void | - | - | - | - |
| 16 | The SS indicates on PDCCH (transmitted in Downlink BWP#0) DL DCI format 1_1 with BWP Id (= BWP#1) and transmits a MAC PDU containing RLC PDU on the configured BWP (i.e. Downlink BWP#1). | <-- | MAC PDU | - | - |
| 17 | After 200 ms from step 16, the SS transmits another valid MAC PDU containing RLC PDU in the active BWP (i.e. Downlink BWP#1). | <-- | MAC PDU | - | - |
|  | EXCEPTION: Step 18 describes behaviour that depends on the UE capability. |  |  |  |  |
| 18a | IF (pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto4_FR2 and band under test is FR2) THEN after 400 ms from step 17, the SS indicates on PDCCH (transmitted in Downlink BWP#1 an UL DCI format 0_1 with BWP Id BWP#1 and allocates an UL Grant, sufficient for loopback of the RLC SDUs from step 16 and step 17. | <-- | UL Grant | - | - |
| 18b | IF pc_bwp_sameNumerology_upto4_FR1_FDD and band under test is FR1 FDD THEN after 400 ms from step 17, the SS indicates on PDCCH (transmitted in Downlink BWP#1 an UL DCI format 0_1 with BWP Id BWP#1 and allocates an UL Grant, sufficient for loopback of the RLC SDUs from step 16 and step 17. | <-- | UL Grant | - | - |
| 18c | IF (pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto2_FR2 and band under test is FR2) THEN after 400 ms from step 17, the SS indicates on PDCCH (transmitted in Downlink BWP#1 an UL DCI format 0_1 with BWP Id BWP#1 and allocates an UL Grant, sufficient for loopback of the RLC SDUs from step 16 and step 17. | <-- | UL Grant | - | - |
| 18d | IF pc_bwp_sameNumerology_upto2_FR1_FDD and band under test is FR1 FDD THEN after 400 ms from step 17, the SS indicates on PDCCH (transmitted in Downlink BWP#1 an UL DCI format 0_1 with BWP Id BWP#1 and allocates an UL Grant, sufficient for loopback of the RLC SDUs from step 16 and step 17. | <-- | UL Grant | - | - |
| - | EXCEPTION: Step 19 describes behaviour that depends on the UE capability. | - | - | - | - |
| 19a | IF (pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto4_FR2 and band under test is FR2) THEN Check: Does the UE transmit a MAC PDU containing both RLC SDUs in the active BWP (i.e. Uplink BWP#1)? (Note 5) | --> | MAC PDU | 6 | P |
| 19b | IF pc_bwp_sameNumerology_upto4_FR1_FDD and band under test is FR1 FDD THEN Check: Does the UE transmit a MAC PDU containing both RLC SDUs in the active BWP (i.e. Uplink BWP#1)? (Note 5) | --> | MAC PDU | 6 | P |
| 19c | IF (pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto2_FR2 and band under test is FR2) THEN Check: Does the UE transmit a MAC PDU containing both RLC SDUs in the active BWP (i.e. Uplink BWP#1)? (Note 5) | --> | MAC PDU | 6 | P |
| 19d | IF pc_bwp_sameNumerology_upto2_FR1_FDD and band under test is FR1 FDD THEN Check: Does the UE transmit a MAC PDU containing both RLC SDUs in the active BWP (i.e. Uplink BWP#1)? (Note 5) | --> | MAC PDU | 6 | P |
| 19A | SS transmits a MAC PDU containing an RLC STATUS PDU acknowledging the reception of the AMD PDU in step 19. | <-- | MAC PDU (RLC STATUS PDU) | - |  |
| - | EXCEPTION: Step 20 describes behaviour that depends on the UE capability. | - | - | - | - |
| 20a | IF (pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto4_FR2 and band under test is FR2) THEN the SS waits 1000 ms from step 18 to ensure that the bwp-InactivityTimer expired and then transmits a valid MAC PDU containing RLC PDU in the BWP with defaultDownlinkBWP-Id (Downlink BWP#2) | <-- | MAC PDU | - | - |
| 20b | IF (pc_bwp_sameNumerology_upto4_FR1_FDD and band under test is FR1 FDD) THEN the SS waits 1000 ms from step 18 to ensure that the bwp-InactivityTimer expired and then transmits a valid MAC PDU containing RLC PDU in the BWP with defaultDownlinkBWP-Id (Downlink BWP#2) | <-- | MAC PDU | - | - |
| 20c | IF (pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto2_FR2 and band under test is FR2) THEN the SS waits 1000 ms from step 18 to ensure that the bwp-InactivityTimer expired and then transmits a valid MAC PDU containing RLC PDU in the BWP with defaultDownlinkBWP-Id (Downlink BWP#0) | <-- | MAC PDU | - | - |
| 20d | IF pc_bwp_sameNumerology_upto2_FR1_FDD and band under test is FR1 FDD THEN the SS waits 1000 ms from step 18 to ensure that the bwp-InactivityTimer expired and then transmits a valid MAC PDU containing RLC PDU in the BWP with defaultDownlinkBWP-Id (Downlink BWP#0) | <-- | MAC PDU | - | - |
| - | EXCEPTION: Step 21 describes behaviour that depends on the UE capability. | - | - | - | - |
| 21a | IF (pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto4_FR2 and band under test is FR2) THEN the SS allocates (transmitted in the defaultDownlinkBWP, i.e. Downlink BWP#2) an UL Grant (with DCI indicating BWP#2) | <-- | UL Grant | - | - |
| 21b | IF pc_bwp_sameNumerology_upto4_FR1_FDD and band under test is FR1 FDD THEN the SS allocates (transmitted in the defaultDownlinkBWP, i.e. Downlink BWP#2) an UL Grant (with DCI indicating BWP#2) | <-- | UL Grant | - | - |
| 21c | IF (pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto2_FR2 and band under test is FR2) THEN the SS allocates (transmitted in the defaultDownlinkBWP, i.e. Downlink BWP#0) an UL Grant (with DCI indicating BWP#0) | <-- | UL Grant | - | - |
| 21d | IF pc_bwp_sameNumerology_upto2_FR1_FDD and band under test is FR1 FDD THEN the SS allocates (transmitted in the defaultDownlinkBWP, i.e. Downlink BWP#0) an UL Grant (with DCI indicating BWP#0) | <-- | UL Grant | - | - |
| - | EXCEPTION: Step 22 describes behaviour that depends on the UE capability. | - | - | - | - |
| 22a | IF (pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto4_FR2 and band under test is FR2) THEN Check: Does the UE transmit a MAC PDU in Uplink BWP#2 (Note 5) | --> | MAC PDU | 5 | P |
| 22b | IF pc_bwp_sameNumerology_upto4_FR1_FDD and band under test is FR1 FDD THEN Check: Does the UE transmit a MAC PDU in Uplink BWP #2 (Note 5) | --> | MAC PDU | 5 | P |
| 22c | IF (pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD) or (pc_bwp_sameNumerology_upto2_FR2 and band under test is FR2) THEN Check: Does the UE transmit a MAC PDU in Uplink BWP#0 (Note 5) | --> | MAC PDU | 5 | P |
| 22d | IF pc_bwp_sameNumerology_upto2_FR1_FDD and band under test is FR1 FDD THEN Check: Does the UE transmit a MAC PDU in Uplink BWP#0 (Note 5) | --> | MAC PDU | 5 | P |
| 22A | SS transmits a MAC PDU containing an RLC STATUS PDU acknowledging the reception of the AMD PDU in step 22. | <-- | MAC PDU (RLC STATUS PDU) | - |  |
| Note 1: For EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration. Note 2: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. Note 3: In paired spectrum (= FDD), the switching of Downlink BWP and Uplink BWP can happen independently. Whereas in TDD, the switching of BWP for Downlink and Uplink is always at the same time instance. Note 4: After the preamble the UE is in RRC_CONNECTED, therefore SRBs and DRBs are already established. The RRCReconfiguration message in step 0 shall not contain any elements like e.g. "rlc-BearerToAddModList" whose value(s) remain unchanged since the preamble. The sole purpose of the RRCReconfiguration message in step 0 is to configure BWPs and related fields for switching of BWPs. Note 5: When the UE does not use the expected BWP for the UL transmission the SS shall not receive the data what implicitly fails the test case. |  |  |  |  |  |

7.1.1.8.1.3.3	Specific message contents

Table 7.1.1.8.1.3.3-0: Conditions for specific message contents

| Condition | Explanation |
| --- | --- |
| BWP#1 | Bandwidth part 1 |
| BWP#2 | Bandwidth part 2 |
| BWP#3 | Bandwidth part 3 |

Table 7.1.1.8.1.3.3-0A: PDSCH-TimeDomainResourceAllocationList (Preamble)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-103 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-TimeDomainResourceAllocationList ::= SEQUENCE(SIZE(1..maxNrofDL-Allocations)) OF PDSCH-TimeDomainResourceAllocation { | 2 entries |  |  |
| PDSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  | entry 1 |  |
| k0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | S=2, L=12 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  | entry 2 |  |
| k0 | 1 |  | pc_bwp_SwitchingDelay_Type1 AND SCS_15KHz |
|  | 6 |  | pc_bwp_SwitchingDelay_Type2 AND SCS_15KHz |
|  | 2 |  | pc_bwp_SwitchingDelay_Type1 AND SCS_30KHz |
|  | 6 |  | pc_bwp_SwitchingDelay_Type2 AND SCS_30KHz |
|  | 3 |  | pc_bwp_SwitchingDelay_Type1 AND SCS_60KHz |
|  | 9 |  | pc_bwp_SwitchingDelay_Type2 AND SCS_60KHz |
|  | 6 |  | pc_bwp_SwitchingDelay_Type1 AND SCS_120KHz |
|  | 18 |  | pc_bwp_SwitchingDelay_Type2 AND SCS_120KHz |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | Start symbol(S)=2, Length(L)=12 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.1.3.3-0B: SchedulingRequest-Config (Preamble)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-155 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequestConfig ::= SEQUENCE { |  |  |  |
| schedulingRequestToAddModList SEQUENCE (SIZE(1..maxNrofSR-ConfigPerCellGroup)) OF SchedulingRequestToAddMod { | 1 entry |  |  |
| SchedulingRequestToAddMod[1] SEQUENCE { |  | entry 1 |  |
| sr-TransMax | n64 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.1.3.3-1: RRCReconfiguration (step 0)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 (see also Note 4 in Table 7.1.1.8.1.3.2-1) |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| secondaryCellGroup |  | CellGroupConfig |  | EN-DC |
| nonCriticalExtension SEQUENCE { |  |  |  | NR |
| masterCellGroup |  | CellGroupConfig |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.8.1.3.3-1A: CellGroupConfig (Table 7.1.1.8.1.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| mac-CellGroupConfig | Not present |  |  |
| physicalCellGroupConfig | Not present |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| servCellIndex | Not present |  |  |
|  | ServCellIndex |  | EN-DC |
| reconfigurationWithSync | Not present |  |  |
| rlf-TimersAndConstants | Not present |  |  |
| spCellConfigDedicated | ServingCellConfig-Dedicated | Table 7.1.1.8.1.3.3-2 |  |
| } |  |  |  |
| reportUplinkTxDirectCurrent | true |  |  |
| } |  |  |  |

Table 7.1.1.8.1.3.3-2: ServingCellConfig-Dedicated (Table 7.1.1.8.1.3.3-1A)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| tdd-UL-DL-ConfigurationDedicated | Not present |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) BWP-Downlink { | 3 entries |  |  |
| BWP-Downlink[1] | BWP-Downlink-BWP-N with condition BWP#1 | entry 1 |  |
| BWP-Downlink[2] | BWP-Downlink-BWP-N with condition BWP#2 | entry 2 | pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD |
| BWP-Downlink[3] | BWP-Downlink-BWP-N with condition BWP#3 | entry 3 | pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 |  |  |
| bwp-InactivityTimer | ms750 |  |  |
| defaultDownlinkBWP-Id | 2 |  | pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD |
|  | 0 |  | pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkDedicated |  |  |
| uplinkBWP-ToReleaseList | Not present |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Uplink { | 3 entries |  |  |
| BWP-Uplink[1] | BWP-Uplink-BWP-N with condition BWP#1 | entry 1 |  |
| BWP-Uplink[2] | BWP-Uplink-BWP-N with condition BWP#2 | entry 2 | pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD |
| BWP-Uplink[3] | BWP-Uplink-BWP-N with condition BWP#3 | entry 3 | pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD |
| } |  |  |  |
| firstActiveUplinkBWP-Id | 1 |  |  |
| pusch-ServingCellConfig | Not present |  |  |
| } |  |  |  |
| pdcch-ServingCellConfig | Not present |  |  |
| pdsch-ServingCellConfig | Not present |  |  |
| csi-MeasConfig | Not present |  |  |
| } |  |  |  |

Table 7.1.1.8.1.3.3-2A: BWP-Downlink-BWP-N (Table 7.1.1.8.1.3.3-2 and Table 7.1.1.8.1.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Downlink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 |  | BWP#1 |
|  | 2 |  | BWP#2 |
|  | 3 |  | BWP#3 |
| bwp-Common SEQUENCE { |  |  |  |
| genericParameters SEQUENCE { |  |  |  |
| locationAndBandwidth | 3850 | Note 6 | BWP#1,2,3 and 3MHz |
|  | 6600 | Note 2 | BWP#1,2,3 and 5MHz |
|  | 6325 | Note 5 | BWP#1,2,3 and 10MHz and SCS30 |
|  | 7975 | Note 3 | BWP#1 |
|  | 12925 | Note 3 | BWP#2 |
|  | 13475 | Note 3 | BWP#3 |
|  | 17875 | Note 4 | BWP#1 and 100MHz |
|  | 14575 | Note 4 | BWP#2 and 100MHz |
|  | 16225 | Note 4 | BWP#3 and 100MHz |
| } |  |  |  |
| pdcch-ConfigCommon | Not present | no cell specific configuration for dedicated BWP |  |
| pdsch-ConfigCommon | Not present | no cell specific configuration for dedicated BWP |  |
| } |  |  |  |
| bwp-Dedicated SEQUENCE { |  |  |  |
| pdcch-Config CHOICE { |  |  |  |
| setup | PDCCH-Config-BWP-N with condition BWP#N |  |  |
| } |  |  |  |
| pdsch-Config CHOICE { |  |  |  |
| setup | PDSCH-Config-BWP-N with condition BWP#N |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| Note 1:Void Note 2: According to TS 38.214 [21] clause 5.1.2.2.2 with =275, LRBs=25 and RBStart=0 for BWP#1,2,3 Note 3: According to TS 38.214 [21] clause 5.1.2.2.2 with =275, LRBs=30,48,50 and RBStart=0 for BWP#1,2,3 Note 4: According to TS 38.214 [21] clause 5.1.2.2.2 with =275, LRBs=66,54,60 and RBStart=0 for BWP#1,2,3 Note 5: According to TS 38.214 [21] clause 5.1.2.2.2 with =275, LRBs=24 and RBStart=0 for BWP#1,2,3 Note 6: According to TS 38.214 [21] clause 5.1.2.2.2 with =275, LRBs=15 and RBStart=0 for BWP#1,2,3 |  |  |  |

| Condition | Explanation |
| --- | --- |
| 3MHz | According to TS 38.508-1 [4] clause 6.2.3.1 with CBW=3Mhz |
| 5MHz | According to TS 38.508-1 [4] clause 6.2.3.1 with CBW=5Mhz |
| 10MHz | According to TS 38.508-1 [4] clause 6.2.3.1 with CBW=10Mhz |
| 100MHz | According to TS 38.508-1 [4] clause 6.2.3.1 with CBW=100Mhz |

Table 7.1.1.8.1.3.3-2B: PDCCH-Config-BWP-N (Table 7.1.1.8.1.3.3-2A)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-95 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-Config::= SEQUENCE { |  |  |  |
| controlResourceSetToAddModList SEQUENCE (SIZE (1..3)) OF ControlResourceSet { | 1 entry |  |  |
| ControlResourceSet[1] | ControlResourceSet-BWP-N with condition BWP#N | entry 1 |  |
| } |  |  |  |
| searchSpacesToAddModList SEQUENCE (SIZE (1..10)) OF SearchSpace { | 2 entries |  |  |
| SearchSpace[1] | SearchSpace-BWP-N with condition BWP#N | entry 1 |  |
| SearchSpace[2] | SearchSpace-CSS-BWP-N with condition BWP#N | entry 2 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.1.3.3-2C: PDSCH-Config-BWP-N (Table 7.1.1.8.1.3.3-2A)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-100 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| pdsch-TimeDomainAllocationList CHOICE { |  |  |  |
| setup | PDSCH-TimeDomainResourceAllocationList |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.1.3.3-2CA: PDSCH-TimeDomainResourceAllocationList (Table 7.1.1.8.1.3.3-2C)

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-TimeDomainAllocationList ::= SEQUENCE { | 2 entries |  |  |
| PDSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  | entry 1 |  |
| k0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | Start symbol(S)=2, Length(L)=12 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  | entry 2 |  |
| k0 | 1 |  | pc_bwp_SwitchingDelay_Type1 AND SCS_15KHz |
|  | 6 |  | pc_bwp_SwitchingDelay_Type2 AND SCS_15KHz |
|  | 2 |  | pc_bwp_SwitchingDelay_Type1 AND SCS_30KHz |
|  | 6 |  | pc_bwp_SwitchingDelay_Type2 AND SCS_30KHz |
|  | 3 |  | pc_bwp_SwitchingDelay_Type1 AND SCS_60KHz |
|  | 9 |  | pc_bwp_SwitchingDelay_Type2 AND SCS_60KHz |
|  | 6 |  | pc_bwp_SwitchingDelay_Type1 AND SCS_120KHz |
|  | 18 |  | pc_bwp_SwitchingDelay_Type2 AND SCS_120KHz |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | Start symbol(S)=2, Length(L)=12 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.1.3.3-2D: ControlResourceSet-BWP-N (Table 7.1.1.8.1.3.3-2B)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 9 |  | BWP#1 |
|  | 10 |  | BWP#2 |
|  | 11 |  | BWP#3 |
| frequencyDomainResources | 10000000 00000000 00000000 00000000 00000000 00000 | CORESET to use the least significant 6 RBs of each BWP |  |
| duration | 2 | SearchSpace duration of 2 symbols |  |
| } |  |  |  |

Table 7.1.1.8.1.3.3-2E: SearchSpace-BWP-N (Table 7.1.1.8.1.3.3-2B)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-162 with condition USS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 37 |  | BWP#1 |
|  | 38 |  | BWP#2 |
|  | 39 |  | BWP#3 |
| controlResourceSetId | 9 |  | BWP#1 |
|  | 10 |  | BWP#2 |
|  | 11 |  | BWP#3 |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n1 |  |  |
| aggregationLevel4 | n0 |  |  |
| aggregationLevel8 | n0 |  |  |
| aggregationLevel16 | n0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.1.3.3-2F: BWP-Uplink-BWP-N (Table 7.1.1.8.1.3.3-2 and Table 7.1.1.8.1.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Uplink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 |  | BWP#1 |
|  | 2 |  | BWP#2 |
|  | 3 |  | BWP#3 |
| bwp-Common SEQUENCE { |  |  |  |
| genericParameters SEQUENCE { |  |  |  |
| locationAndBandwidth | 3850 | Note 6 | BWP#1,2,3 and 3MHz |
|  | 6600 | Note 2 | BWP#1,2,3 and 5MHz |
|  | 6325 | Note 5 | BWP#1,2,3 and 10MHz and SCS30 |
|  | 7975 | Note 3 | BWP#1 |
|  | 12925 | Note 3 | BWP#2 |
|  | 13475 | Note 3 | BWP#3 |
|  | 17875 | Note 4 | BWP#1 and 100MHz |
|  | 14575 | Note 4 | BWP#2 and 100MHz |
|  | 16225 | Note 4 | BWP#3 and 100MHz |
| } |  |  |  |
| rach-ConfigCommon | Not present | No cell specific configuration for dedicated BWP |  |
| pusch-ConfigCommon CHOICE { |  |  |  |
| setup | PUSCH-ConfigCommon-BWP-N |  |  |
| } |  |  |  |
| pucch-ConfigCommon CHOICE { |  |  |  |
| setup | PUCCH-ConfigCommon-BWP-N |  |  |
| } |  |  |  |
| } |  |  |  |
| bwp-Dedicated SEQUENCE { |  |  |  |
| pucch-Config CHOICE { |  |  |  |
| setup | PUCCH-Config-BWP-N |  |  |
| } |  |  |  |
| pusch-Config CHOICE { |  |  |  |
| setup | PUSCH-Config-BWP-N |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| Note 1:Void Note 2: According to TS 38.214 [21] clause 5.1.2.2.2 with =275, LRBs=25 and RBStart=0 for BWP#1,2,3 Note 3: According to TS 38.214 [21] clause 5.1.2.2.2 with =275, LRBs=30,48,50 and RBStart=0 for BWP#1,2,3 Note 4: According to TS 38.214 [21] clause 5.1.2.2.2 with =275, LRBs=66,54,60 and RBStart=0 for BWP#1,2,3 Note 5: According to TS 38.214 [21] clause 5.1.2.2.2 with =275, LRBs=24 and RBStart=0 for BWP#1,2,3 Note 6: According to TS 38.214 [21] clause 5.1.2.2.2 with =275, LRBs=15 and RBStart=0 for BWP#1,2,3 |  |  |  |

| Condition | Explanation |
| --- | --- |
| 3MHz | According to TS 38.508-1 [4] clause 6.2.3.1 with CBW=3Mhz |
| 5MHz | According to TS 38.508-1 [4] clause 6.2.3.1 with CBW=5Mhz |
| 10MHz | According to TS 38.508-1 [4] clause 6.2.3.1 with CBW=10Mhz |
| 100MHz | According to TS 38.508-1 [4] clause 6.2.3.1 with CBW=100Mhz |

Table 7.1.1.8.1.3.3-2G: PUCCH-Config-BWP-N (Table 7.1.1.8.1.3.3-2F)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-112 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-Config ::= SEQUENCE { |  |  |  |
| resourceSetToAddModList SEQUENCE (SIZE (1..4)) OF PUCCH-ResourceSet { | 1 entry |  |  |
| PUCCH-ResourceSet[1] SEQUENCE { |  | entry 1 |  |
| pucch-ResourceSetId | 0 |  |  |
| resourceList SEQUENCE (SIZE (1..32)) OF PUCCH-ResourceId { | 1 entry |  |  |
| PUCCH-ResourceId[1] | 0 | entry 1 |  |
| } |  |  |  |
| maxPayloadSize | 256 |  |  |
| } |  |  |  |
| } |  |  |  |
| resourceToAddModList SEQUENCE (SIZE (1..128)) OF PUCCH-Resource { | 1 entry |  |  |
| PUCCH-Resource[1] SEQUENCE { |  | entry 1 |  |
| pucch-RessourceId | 0 |  |  |
| startingPRB | 0 |  | BWP#1 |
|  | 0 |  | BWP#2 |
|  | 0 |  | BWP#3 |
| intraSlotFrequencyHopping | disabled |  |  |
| secondHopPRB | Not Present |  |  |
| format CHOICE { |  |  |  |
| format0 SEQUENCE { |  |  |  |
| initialCyclicShift | 0 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 10 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| schedulingRequestResourceToAddModList SEQUENCE (SIZE (1..maxNrofSR-Resources)) OF SchedulingRequestResourceConfig { | 1 entry |  |  |
| SchedulingRequestResourceConfig[1] | SchedulingRequestResourceConfig | entry 1 Table 7.1.1.8.1.3.3-2K |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.1.3.3-2H: PUSCH-Config-BWP-N (Table 7.1.1.8.1.3.3-2F)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-118 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| pusch-TimeDomainAllocationList CHOICE { |  |  |  |
| setup | PUSCH-TimeDomainResourceAllocationList |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.1.3.3-2I: PUSCH-TimeDomainResourceAllocationList (Table 7.1.1.8.1.3.3-2F and Table 7.1.1.8.1.3.3-2M)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-122 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-TimeDomainResourceAllocationList ::= SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF PUSCH-TimeDomainResourceAllocation { | 2 entries |  |  |
| PUSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  | entry 1 |  |
| k2 | 4 |  |  |
|  | 6 |  | (pc_bwp_SwitchingDelay_Type2 AND SCS_30KHz) OR (pc_bwp_SwitchingDelay_Type1 AND SCS_120KHz) |
|  | 18 |  | (pc_bwp_SwitchingDelay_Type2 AND SCS_120KHz) |
| } |  |  |  |
| PUSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  | entry 2 |  |
| k2 | Not present |  |  |
|  | 2 |  | FR1 AND SCS15 |
|  | 6 |  | FR1 AND SCS30 |
|  | 3 |  | FR2 |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.1.3.3-2J: Physical layer parameters for DCI format 1_1 (Steps 5-22A, Table 7.1.1.8.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.3.6.1.2.2-1 |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| PUCCH resource indicator | PUCCH-ResourceId = 0 | “000” |  |

Table 7.1.1.8.1.3.3-2K: SchedulingRequestResourceConfig (Table 7.1.1.8.1.3.3-2G)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-157 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequestResourceConfig ::= SEQUENCE { |  |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| sl20 | 9 | With SCS = kHz15 results in repetition every 20 ms | SCS15 AND BWP#1 |
| sl40 | 9 | With SCS = kHz30 results in repetition every 20 ms | SCS30 AND BWP#1 |
| sl80 | 19 | With SCS = kHz60 results in repetition every 20 ms | FR1 AND SCS60 AND BWP#1 |
|  | 9 | With SCS = kHz60 results in repetition every 20 ms | FR2 AND SCS60 AND BWP#1 |
| sl160 | 9 | With SCS = kHz120 results in repetition every 20 ms | SCS120 AND BWP#1 |
| } |  |  |  |
| resource | 0 |  |  |
| } |  |  |  |

Table 7.1.1.8.1.3.3-2L: BWP-UplinkDedicated (Table 7.1.1.8.1.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicated ::= SEQUENCE { |  | BWP-UplinkDedicated ::= SEQUENCE { |  |
| pusch-Config CHOICE { |  |  |  |
| setup | PUSCH-Config |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.1.3.3-2M: PUSCH-Config (Table 7.1.1.8.1.3.3-2L)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-118 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config:: = SEQUENCE { |  |  |  |
| pusch-TimeDomainAllocationList CHOICE { |  |  |  |
| setup | PUSCH-TimeDomainResourceAllocationList | Table 7.1.1.8.1.3.3-2I |  |
| } |  |  |  |

Table 7.1.1.8.1.3.3-3: PUCCH-ConfigCommon-BWP-N (Table 7.1.1.8.1.3.3-2F)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-113 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| pucch-ResourceCommon | Not Present |  |  |
| pucch-GroupHopping | enable |  |  |
| hoppingId | Not present |  |  |
| p0-nominal | Not Present |  |  |
| } |  |  |  |

Table 7.1.1.8.1.3.3-4: SearchSpace-CSS-BWP-N (Table 7.1.1.8.1.3.3-2B)

| Derivation Path: TS 38.508-1[4], Table 4.6.3-162 with condition CSS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 30 |  | BWP#1 |
|  | 31 |  | BWP#2 |
|  | 32 |  | BWP#3 |
| controlResourceSetId | 9 |  | BWP#1 |
|  | 10 |  | BWP#2 |
|  | 11 |  | BWP#3 |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n1 |  |  |
| aggregationLevel4 | n0 |  |  |
| aggregationLevel8 | n0 |  |  |
| aggregationLevel16 | n0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.1.3.3-5: Void

Table 7.1.1.8.1.3.3-6: Void

Table 7.1.1.8.1.3.3-7: Void

Table 7.1.1.8.1.3.3-8: Void

Table 7.1.1.8.1.3.3-9: PUSCH-ConfigCommon-BWP-N (Table 7.1.1.8.1.3.3-2F)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-119 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| groupHoppingEnabledTransformPrecoding | Not present |  |  |
| pusch-TimeDomainAllocationList | Not present |  |  |
| msg3-DeltaPreamble | 1 |  |  |
| p0-NominalWithGrant | -90 |  |  |
| } |  |  |  |

Table 7.1.1.8.1.3.3-10: Void

##### 7.1.1.8.2

##### 7.1.1.8.3	Separate BWP / IDLE / RedCap

7.1.1.8.3.1	Test Purpose (TP)

(1)

with { UE supporting RedCap and in NR RRC_IDLE state }

ensure that {

when { initialDownlinkBWP-RedCap is configured }

then { the UE shall monitor the PDCCH on the BWP configured by initialDownlinkBWP-RedCap }

}

(2)

with { UE supporting RedCap and in NR RRC_IDLE state }

ensure that {

when { initialUplinkBWP-RedCap is configured and the UE needs to perform the Random Access procedure }

then { the UE shall perform the Random Access procedure by using the BWP configured by initialUplinkBWP-RedCap }

}

7.1.1.8.3.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.321, clause 5.15.1. Unless otherwise stated these are Rel-17 requirements.

[TS 38.321, clause 5.15.1]

A RedCap UE may be configured with a RedCap-specific initial UL BWP in initialUplinkBWP-RedCap, as specified in TS 38.331 [5].

Upon initiation of the Random Access procedure on a Serving Cell, after the selection of carrier for performing Random Access procedure as specified in clause 5.1.1, the MAC entity shall for the selected carrier of this Serving Cell:

1>	if PRACH occasions are not configured for the active UL BWP:

2>	if the UE is a RedCap UE; and

2>	if initialUplinkBWP-RedCap is configured:

3>	switch the active UL BWP to BWP configured by initialUplinkBWP-RedCap.

2>	else:

3>	switch the active UL BWP to BWP indicated by initialUplinkBWP.

2>	if the Serving Cell is an SpCell:

3>	if the UE is a RedCap UE; and

3>	if initialDownlinkBWP-RedCap is configured:

4>	switch the active DL BWP to BWP configured by initialDownlinkBWP-RedCap.

3>	else:

4>	switch the active DL BWP to BWP indicated by initialDownlinkBWP.

1>	else:

2>	if the Serving Cell is an SpCell:

3>	if the active DL BWP does not have the same bwp-Id as the active UL BWP:

4>	switch the active DL BWP to the DL BWP with the same bwp-Id as the active UL BWP.

1>	stop the bwp-InactivityTimer associated with the active DL BWP of this Serving Cell, if running.

1>	if the Serving Cell is SCell:

2>	stop the bwp-InactivityTimer associated with the active DL BWP of SpCell, if running.

1>	perform the Random Access procedure on the active DL BWP of SpCell and active UL BWP of this Serving Cell.

If the MAC entity receives a PDCCH for BWP switching of a Serving Cell, the MAC entity shall:

1>	if there is no ongoing Random Access procedure associated with this Serving Cell; or

1>	if the ongoing Random Access procedure associated with this Serving Cell is successfully completed upon reception of this PDCCH addressed to C-RNTI (as specified in clauses 5.1.4, 5.1.4a, and 5.1.5):

2>	cancel, if any, triggered consistent LBT failure for this Serving Cell;

2>	perform BWP switching to a BWP indicated by the PDCCH.

…

Upon reception of RRC (re-)configuration for BWP switching for a Serving Cell, cancel any triggered LBT failure in this Serving Cell.

The MAC entity shall for each activated Serving Cell configured with bwp-InactivityTimer:

1>	if the defaultDownlinkBWP-Id is configured, and the active DL BWP is not the BWP indicated by the defaultDownlinkBWP-Id, and the active DL BWP is not the BWP indicated by the dormantBWP-Id if configured; or

1>	if the defaultDownlinkBWP-Id is not configured, and if the UE is not a RedCap UE, and the active DL BWP is not the initialDownlinkBWP, and the active DL BWP is not the BWP indicated by the dormantBWP-Id if configured; or

1>	if the defaultDownlinkBWP-Id is not configured and if the UE is a RedCap UE, and initialDownlinkBWP-RedCap is not configured, and the active DL BWP is not the initialDownlinkBWP, and the active DL BWP is not the BWP indicated by the dormantBWP-Id if configured; or

1>	if the defaultDownlinkBWP-Id is not configured and if the UE is a RedCap UE, and initialDownlinkBWP-RedCap is configured, the active DL BWP is not the initialDownlinkBWP-RedCap, and the active DL BWP is not the BWP indicated by the dormantBWP-Id if configured:

2>	if a PDCCH addressed to C-RNTI or CS-RNTI indicating downlink assignment or uplink grant is received on the active BWP; or

2>	if a PDCCH addressed to G-RNTI or G-CS-RNTI configured for multicast indicating downlink assignment is received on the active BWP; or

2>	if a PDCCH addressed to C-RNTI or CS-RNTI indicating downlink assignment or uplink grant is received for the active BWP; or

2>	if a MAC PDU is transmitted in a configured uplink grant and LBT failure indication is not received from lower layers; or

2>	if a MAC PDU is received in a configured downlink assignment for unicast or MBS multicast:

3>	if there is no ongoing Random Access procedure associated with this Serving Cell; or

3>	if the ongoing Random Access procedure associated with this Serving Cell is successfully completed upon reception of this PDCCH addressed to C-RNTI (as specified in clauses 5.1.4, 5.1.4a and 5.1.5):

4>	start or restart the bwp-InactivityTimer associated with the active DL BWP.

2>	if the bwp-InactivityTimer associated with the active DL BWP expires:

3>	if the defaultDownlinkBWP-Id is configured:

4>	perform BWP switching to a BWP indicated by the defaultDownlinkBWP-Id.

3>	else:

4>	if the UE is a RedCap UE; and

4>	if initialDownlinkBWP-RedCap is configured:

5>	perform BWP switching to the initialDownlinkBWP-RedCap.

4>	else:

5>	perform BWP switching to the initialDownlinkBWP.

NOTE:	If a Random Access procedure is initiated on an SCell, both this SCell and the SpCell are associated with this Random Access procedure.

1>	if a PDCCH for BWP switching is received, and the MAC entity switches the active DL BWP:

2>	if the defaultDownlinkBWP-Id is configured, and the MAC entity switches to the DL BWP which is not indicated by the defaultDownlinkBWP-Id and is not indicated by the dormantBWP-Id if configured; or

2>	if the defaultDownlinkBWP-Id is not configured, and the MAC entity switches to the DL BWP which is not the initialDownlinkBWP and is not indicated by the dormantBWP-Id if configured:

3>	start or restart the bwp-InactivityTimer associated with the active DL BWP.

Upon initiation of the Random Access procedure, after selection of the carrier for performing Random Access procedure as specified in clause 5.1.1, if the UE is a RedCap UE in RRC_IDLE or RRC_INACTIVE mode, the MAC entity shall:

1>	if initialUplinkBWP-RedCap is configured:

2>	perform the Random Access procedure as specified in clause 5.1 by using the BWP configured by initialUplinkBWP-RedCap.

1>	else:

2>	perform the Random Access procedure as specified in clause 5.1 by using the BWP configured by initialUplinkBWP.

1>	if initialDownlinkBWP-RedCap is configured:

2>	monitor the PDCCH on the BWP configured by initialDownlinkBWP-RedCap.

1>	else:

2>	monitor the PDCCH on the BWP configured by initialDownlinkBWP.

7.1.1.8.3.3	Test description

7.1.1.8.3.3.1	Pre-test conditions

System Simulator:

-	NR Cell 2.

UE:

-	None

Preamble:

-	The UE is in NR RRC_Idle mode (state 1N-A) on NR Cell 2 according to 38.508-1 [4] Table 4.4A.2-1.

7.1.1.8.3.3.2	Test procedure sequence

Table 7.1.1.8.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes the SIB1 of NR Cell 2 to configure initialDownlinkBWP-RedCap and initialUplinkBWP-RedCap. | - | - | - | -- |
| 2 | The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. | <-- | PDCCH (DCI 1_0): Short Message | - | - |
| 3 | Wait for 2.1* modification period second for the UE to receive new system information. (Note 1) | - | - | - | - |
| 4 | The SS transmits a Paging message within the BWP configured by initialDownlinkBWP-RedCap. | <-- | NR RRC: Paging | - | - |
| 5 | Check: Does the UE transmit an RRCSetupRequest message by setting LCID to 35? | --> | NR RRC: RRCSetupRequest | 1 | P |
| 6 | The SS transmits an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 7 | Check: Does the UE transmit HARQ ACK for step 6 on PUCCH configured by initialUplinkBWP-RedCap? | --> | HARQ ACK | 2 | P |
| 8-13a1 | Steps 4-9a1 of Generic procedure as specified in TS 38.508-1 [4] Table 4.9.4.2.2-1 are performed. | - | - | - | - |
| Note 1: The modification period, expressed in number of radio frames = modificationPeriodCoeff * defaultPagingCycle. |  |  |  |  |  |

7.1.1.8.3.3.3	Specific message contents

Table 7.1.1.8.3.3.3-1: SIB1 (step 1, Table 7.1.1.8.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| servingCellConfigCommon |  | ServingCellConfigCommonSIB | Table 7.1.1.8.3.3.3-2 |  |
| } |  |  |  |  |

Table 7.1.1.8.3.3.3-2: ServingCellConfigCommonSIB (Table 7.1.1.8.3.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| downlinkConfigCommon | DownlinkConfigCommonSIB | Table 7.1.1.8.3.3.3-3 |  |
| uplinkConfigCommon-v1700 ::= SEQUENCE { |  |  |  |
| initialUplinkBWP-RedCap-r17 | BWP-UplinkCommon | Table 7.1.1.8.3.3.3-8 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.3.3.3-3: DownlinkConfigCommonSIB (Table 7.1.1.8.3.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-53 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DownlinkConfigCommonSIB ::= SEQUENCE { |  |  |  |
| pei-Config-r17 | Not present |  |  |
| initialDownlinkBWP-RedCap-r17 | BWP-DownlinkCommon-RedCap | Table 7.1.1.8.3.3.3-4 |  |
| } |  |  |  |

Table 7.1.1.8.3.3.3-4: BWP-DownlinkCommon-RedCap (Table 7.1.1.8.3.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-10 with condition InitialBWP_SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkCommon ::= SEQUENCE { |  |  |  |
| pdcch-ConfigCommon CHOICE { |  |  |  |
| setup | PDCCH-ConfigCommon | Table 7.1.1.8.3.3.3-5 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.3.3.3-5: PDCCH-ConfigCommon (Table 7.1.1.8.3.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-96 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| commonSearchSpaceList SEQUENCE (SIZE (1..4)) OF SearchSpace { | 2 entries |  |  |
| SearchSpace[1] | SearchSpace with condition CSS as specified in TS 38.508-1 [4] Table 4.6.3-162 | entry 1 |  |
| SearchSpace[2] | SearchSpace | entry 2, Table 7.1.1.8.3.3.3-6 |  |
| } |  |  |  |
| searchSpaceOtherSystemInformation | Not present |  |  |
| pagingSearchSpace | 4 |  |  |
| } |  |  |  |

Table 7.1.1.8.3.3.3-6: SearchSpace (Table 7.1.1.8.3.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-162 with condition CSS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 4 |  |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl10 | 2 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.3.3.3-7: Void

Table 7.1.1.8.3.3.3-8: BWP-UplinkCommon (Table 7.1.1.8.3.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkCommon ::= SEQUENCE { |  |  |  |
| pucch-ConfigCommon CHOICE { |  |  |  |
| setup | PUCCH-ConfigCommon | Table 7.1.1.8.3.3.3-9 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.3.3.3-9: PUCCH-ConfigCommon (Table 7.1.1.8.3.3.3-8)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-113 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| pucch-ResourceCommon | Not present |  |  |
| nrofPRBs | Not present |  |  |
| intra-SlotFH-r17 | fromLowerEdge |  |  |
| pucch-ResourceCommon-RedCap-r17 | 1 |  |  |
| additionalPRBOffset-r17 | n2 |  |  |
| } |  |  |  |

##### 7.1.1.8.4	Separate BWP / RedCap-specific initial DL BWP without CORESET#0 / NCD-SSB

7.1.1.8.4.1	Test Purpose (TP)

(1)

with { UE supporting RedCap and in NR RRC_IDLE state }

ensure that {

when { initialDownlinkBWP-RedCap does not contain the entire CORESET#0 }

then { UE uses this BWP for receiving DL messages during initial access and after initial access }

}

(2)

with { UE in NR RRC_CONNECTED and the active BWP is RedCap-specific initial downlink BWP and this BWP does not include CD-SSB and the entire CORESET#0 }

ensure that {

when { NCD-SSB is configured for a RedCap UE in RRCSetup }

then { UE replies RRCSetupComplete }

}

(3)

with { UE in NR RRC_CONNECTED and the active BWP is RedCap-specific initial downlink BWP and this BWP does not include CD-SSB and the entire CORESET#0 }

ensure that {

when { SS sends RRCRelease to trigger UE move to RRC_IDLE }

then { UE shall monitor paging in the initial DL BWP that includes CORESET#0 }

}

7.1.1.8.4.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.300, clause 16.13.5, TS 38.321, clause 5.15.1 and TS 38.331, clause 6.3.2. Unless otherwise stated these are Rel-17 requirements.

[TS 38.321, clause 5.15.1]

Upon initiation of the Random Access procesdure, after selection of the carrier for performing Random Access procedure as specified in clause 5.1.1, if the UE is a RedCap UE in RRC_IDLE or RRC_INACTIVE mode, the MAC entity shall:

1>	if initialUplinkBWP-RedCap is configured for the selected carrier:

2>	perform the Random Access procedure as specified in clause 5.1 by using the BWP configured by initialUplinkBWP-RedCap.

1>	else:

2>	perform the Random Access procedure as specified in clause 5.1 by using the BWP configured by initialUplinkBWP.

1>	if initialDownlinkBWP-RedCap is configured:

2>	if the Random Access procedure was initiated for SI request (as specified in TS 38.331 [5]) and the Random Access Resources for SI request have been explicitly provided by RRC, and if the selected carrier is SUL carrier:

3>	monitor the PDCCH on the BWP configured by initialDownlinkBWP.

2>	else:

3>	monitor the PDCCH on the BWP configured by initialDownlinkBWP-RedCap.

1>	else:

2>	monitor the PDCCH on the BWP configured by initialDownlinkBWP.

[TS 38.300, clause 16.13.5]

A RedCap UE may be configured with multiple NCD-SSBs provided that each BWP is configured with at most one SSB. NCD-SSB may be configured for a RedCap UE in RRC_CONNECTED to perform RLM, BFD, and RRM measurements when the active BWP does not contain CD-SSB.

[TS 38.331, clause 6.3.2]

DownlinkConfigCommonSIB field descriptions

initialDownlinkBWP-RedCap If present, RedCap UEs use this DL BWP instead of initialDownlinkBWP. If the locationAndBandwidth of this BWP contains the entire CORESET#0, the UE applies the locationAndBandwidth upon reception of this field (e.g. to determine the frequency position of signals described in relation to this locationAndBandwidth) but it keeps CORESET#0 until after reception of RRCSetup/RRCResume/RRCReestablishment. Otherwise, i.e., if the locationAndBandwidth of this BWP does not contain the entire CORESET#0, the UE uses this BWP for receiving DL messages during initial access (Msg2, MsgB, Msg4) and after initial access. If absent, RedCap UEs use initialDownlinkBWP provided that it does not exceed the RedCap UE maximum bandwidth (see also clause 5.2.2.4.2).

BWP-DownlinkDedicated field descriptions

nonCellDefiningSSB If configured, the RedCap UE operating in this BWP uses this SSB for the purposes for which it would otherwise have used the cell-defining SSB of the serving cell (e.g. obtaining sync, measurements, RLM). Furthermore, other parts of the BWP configuration that refer to an SSB (e.g. the "SSB" configured in the QCL-Info IE; the "ssb-Index" configured in the RadioLinkMonitoringRS; CFRA-SSB-Resource; PRACH-ResourceDedicatedBFR) refer implicitily to this NCD-SSB. The NCD-SSB has the same values for the properties (e.g., ssb-PositionsInBurst, PCI, ssb-periodicity, ssb-PBCH-BlockPower) of the corresponding CD-SSB apart from the values of the properties configured in the NonCellDefiningSSB-r17 IE.

PDCCH-ConfigCommon field descriptions

commonControlResourceSet An additional common control resource set which may be configured and used for any common or UE-specific search space. If the network configures this field, it uses a ControlResourceSetId other than 0 for this ControlResourceSet. The network configures the commonControlResourceSet in SIB1 so that it is contained in the bandwidth of CORESET#0. If the RedCap-specific initial downlink BWP does not contain the entire CORESET#0, the network configures the commonControlResourceSet in the RedCap-specific initial downlink BWP in SIB1 for RedCap such that it does not have to be contained in the bandwidth of CORESET#0.

pagingSearchSpace ID of the search space for paging (see TS 38.213 [13], clause 10.1). If the field is absent, the UE does not receive paging in this BWP (see TS 38.213 [13], clause 10). This field is absent for the RedCap-specific initial downlink BWP, if it does not include CD-SSB and the entire CORESET#0. In that case, a RedCap UE in RRC_IDLE or RRC_INACTIVE while SDT procedure is not ongoing, shall monitor paging in the initial DL BWP that includes CORESET#0.

ra-SearchSpace ID of the Search space for random access procedure (see TS 38.213 [13], clause 10.1). If the field is absent, the UE does not receive RAR in this BWP. This field is mandatory present in the DL BWP(s) if the conditions described in TS 38.321 [3], clause 5.15 are met.

searchSpaceOtherSystemInformation ID of the Search space for other system information, i.e., SIB2 and beyond (see TS 38.213 [13], clause 10.1). If the field is absent, the UE does not receive other system information in this BWP. This field is absent for the RedCap-specific initial DL BWP, if it does not include CD-SSB and the entire CORESET#0. In that case, a RedCap UE in RRC_IDLE or RRC_INACTIVE shall monitor PDCCH to receive other system information using searchSpaceOtherSystemInformation in the initial DL BWP that includes CD-SSB and the entire CORESET#0.

searchSpaceSIB1 ID of the search space for SIB1 message. In the initial DL BWP of the UE′s PCell, the network sets this field to 0. If the field is absent, the UE does not receive SIB1 in this BWP. (see TS 38.213 [13], clause 10). This field is absent for the RedCap-specific initial DL BWP, if it does not include CD-SSB and the entire CORESET#0. In that case, a RedCap UE in RRC_IDLE or RRC_INACTIVE shall monitor PDCCH to receive SIB1 using searchSpaceSIB1 in the initial DL BWP that includes CD-SSB and the entire CORESET#0.

7.1.1.8.4.3	Test description

7.1.1.8.4.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is configured to use Test frequency NRf1 as specified in TS 38.508-1 [4] Table 6.2.3.1-8 and Table 6.2.3.1-9 using the common highest mandatory UL and DL channel bandwidth and using the default subcarrier spacing specified in TS 38.508-1 [4] clause 6.2.3.1.

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR Cell 1 with SIB1 content as specified in Table 7.1.1.8.4.3.3-1 and SS broadcasts CD-SSB and NCD-SSB. MIB for NCD-SSB is specified in Table 7.1.1.8.4.3.3-0.

UE:

-	None

Preamble:

-	The UE is in state Switched OFF (state 0-A) as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1.

7.1.1.8.4.3.2	Test procedure sequence

Table 7.1.1.8.4.3.2-1: Void

Table 7.1.1.8.4.3.2-2: Void

Table 7.1.1.8.4.3.2-3: Void

Table 7.1.1.8.4.3.2-4: Void

Table 7.1.1.8.4.3.2-5: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Void. | - | - | - | - |
| 2 | Power on the UE. | - | - | - | - |
| 3 | The UE transmits a preamble on PRACH in initialUplinkBWP-RedCap? | --> | PRACH Preamble | - | - |
| 4 | The SS transmits Random Access Response in initialDownlinkBWP-RedCap. | <-- | Random Access Response | - | - |
| 5 | Check: Does the UE transmit a MAC PDU containing an RRCSetupRequest message? | --> | NR RRC: RRCSetupRequest | 1 | P |
| 6 | The SS transmits RRCSetup including NCD-SSB. | <-- | NR RRC: RRCSetup | - | - |
| 7 | Check: Does the UE transmit an RRCSetupComplete message? | --> | NR RRC: RRCSetupComplete | 2 | P |
| 8-23a1 | Steps 5 to 20a1 of the registration procedure described in TS 38.508-1 [4] subclause 4.5.2.2-2 are performed on NR Cell 1. NOTE: The UE performs registration and the RRC connection is released. | - | - | - | - |
| 24 | The SS transmits a Paging message including a matched UE identity. | <-- | NR RRC: Paging | - | - |
| 25 | The UE transmits a preamble on PRACH in initialUplinkBWP-RedCap? | --> | PRACH Preamble | - | - |
| 26 | The SS transmits Random Access Response in initialDownlinkBWP-RedCap. | <-- | Random Access Response | - | - |
| 27 | Check: Does the UE transmit a MAC PDU containing an RRCSetupRequest message? | --> | NR RRC: RRCSetupRequest | 1,3 | P |
| 28 | The SS transmits RRCSetup including NCD-SSB. | <-- | NR RRC: RRCSetup | - | - |
| 29 | Check: Does the UE transmit an RRCSetupComplete message? | --> | NR RRC: RRCSetupComplete | 2 | P |
| 30-34a1 | Steps 5-9a1 of Generic procedure as specified in TS 38.508-1 [4] Table 4.9.4.2.2-1 are performed. | - | - | - | - |

7.1.1.8.4.3.3	Specific message contents

Table 7.1.1.8.4.3.3-0: MIB for NCD-SSB (preamble and all steps, Table 7.1.1.8.4.3.2-5)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-11 with condition NCD-SSB

Table 7.1.1.8.4.3.3-1: SIB1 (preamble and all steps, Table 7.1.1.8.4.3.2-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| servingCellConfigCommon |  | ServingCellConfigCommonSIB | Table 7.1.1.8.4.3.3-2 |  |
| } |  |  |  |  |

Table 7.1.1.8.4.3.3-2: ServingCellConfigCommonSIB (Table 7.1.1.8.4.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| downlinkConfigCommon | DownlinkConfigCommonSIB | Table 7.1.1.8.4.3.3-3 |  |
| uplinkConfigCommon-v1700 SEQUENCE { |  |  |  |
| initialUplinkBWP-RedCap-r17 | BWP-UplinkCommon-RedCap | Table 7.1.1.8.4.3.3-9 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.4.3.3-3: DownlinkConfigCommonSIB (Table 7.1.1.8.4.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-53 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DownlinkConfigCommonSIB ::= SEQUENCE { |  |  |  |
| pei-Config-r17 | Not present |  |  |
| initialDownlinkBWP-RedCap-r17 | BWP-DownlinkCommon-RedCap | Table 7.1.1.8.4.3.3-4 |  |
| } |  |  |  |

Table 7.1.1.8.4.3.3-4: BWP-DownlinkCommon-RedCap (Table 7.1.1.8.4.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-10 with condition InitialBWP_SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkCommon ::= SEQUENCE { |  |  |  |
| genericParameters | BWP-RedCap |  |  |
| pdcch-ConfigCommon CHOICE { |  |  |  |
| setup | PDCCH-ConfigCommon-RedCap | Table 7.1.1.8.4.3.3-5 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.4.3.3-5: PDCCH-ConfigCommon-RedCap (Table 7.1.1.8.4.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-96 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| controlResourceSetZero | Not present |  |  |
| commonControlResourceSet | ControlResourceSet | Table 7.1.1.8.4.3.3-6 |  |
| searchSpaceZero | Not present |  |  |
| commonSearchSpaceList SEQUENCE (SIZE (1..4)) OF SearchSpace { | 1 entry |  |  |
| SearchSpace[1] | SearchSpace | entry 1 Table 7.1.1.8.4.3.3-7 |  |
| } |  |  |  |
| searchSpaceSIB1 | Not present |  |  |
| searchSpaceOtherSystemInformation | Not present |  |  |
| pagingSearchSpace | Not present |  |  |
| ra-SearchSpace | SearchSpaceId with condition CSS |  |  |
| } |  |  |  |

Table 7.1.1.8.4.3.3-6: ControlResourceSet (Table 7.1.1.8.4.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 2 |  |  |
| } |  |  |  |

Table 7.1.1.8.4.3.3-7: SearchSpace (Table 7.1.1.8.4.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-162 with condition CSS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 2 |  |  |
| } |  |  |  |

Table 7.1.1.8.4.3.3-8: BWP-RedCap (Table 7.1.1.8.4.3.3-4 and Table 7.1.1.8.4.3.3-9)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP ::= SEQUENCE { |  |  |  |
| locationAndBandwidth | Set to value of locationAndBandwidth in TS 38.508-1 [4] Table 6.2.3.1-8 |  | FR1 |
|  | Set to value of locationAndBandwidth in TS 38.508-1 [4] Table 6.2.3.1-9 |  | FR2 |
| } |  |  |  |

Table 7.1.1.8.4.3.3-9: BWP-UplinkCommon-RedCap (Table 7.1.1.8.4.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkCommon ::= SEQUENCE { |  |  |  |
| genericParameters | BWP-RedCap | Table 7.1.1.8.4.3.3-8 |  |
| pucch-ConfigCommon CHOICE { |  |  |  |
| setup | PUCCH-ConfigCommon-RedCap | Table 7.1.1.8.4.3.3-10 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.4.3.3-10: PUCCH-ConfigCommon-RedCap (Table 7.1.1.8.4.3.3-9)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-113 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| pucch-ResourceCommon | Not present |  |  |
| pucch-ResourceCommonRedCap-r17 | 0 |  |  |
| } |  |  |  |

Table 7.1.1.8.4.3.3-11: RRCSetup (step 6, Table 7.1.1.8.4.3.2-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-21 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCSetup ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcSetup SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig-RedCap | Table 7.1.1.8.4.3.3-12 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.8.4.3.3-12: CellGroupConfig-RedCap (Table 7.1.1.8.4.3.3-11)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated | ServingCellConfig-RedCap | Table 7.1.1.8.4.3.3-13 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.4.3.3-13: ServingCellConfig-RedCap (Table 7.1.1.8.4.3.3-12)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkDedicated-RedCap | Table 7.1.1.8.4.3.3-14 |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkDedicated-RedCap | Table 7.1.1.8.4.3.3-15 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.4.3.3-14: BWP-DownlinkDedicated-RedCap (Table 7.1.1.8.4.3.3-13)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkDedicated ::= SEQUENCE { |  |  |  |
| pdcch-Config CHOICE { |  |  |  |
| setup | PDCCH-Config | Table 7.1.1.8.4.3.3-16 |  |
| } |  |  |  |
| nonCellDefiningSSB-r17 | NonCellDefiningSSB | Table 7.1.1.8.4.3.3-18 |  |
| } |  |  |  |

Table 7.1.1.8.4.3.3-15: BWP-UplinkDedicated-RedCap (Table 7.1.1.8.4.3.3-13)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicated ::= SEQUENCE { |  |  |  |
| pucch-Config CHOICE { |  |  |  |
| setup | PUCCH-Config | Table 7.1.1.8.4.3.3-19 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.4.3.3-16: PDCCH-Config (Table 7.1.1.8.4.3.3-14)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-95 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-Config ::= SEQUENCE { |  |  |  |
| controlResourceSetToAddModList | Not present |  |  |
| controlResourceSetToReleaseList | Not present |  |  |
| searchSpacesToAddModList SEQUENCE(SIZE (1..10)) OF SearchSpace { | 1 entry |  |  |
| SearchSpace[1] | SearchSpace | entry 1 Table 7.1.1.8.4.3.3-17 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.4.3.3-17: SearchSpace (Table 7.1.1.8.4.3.3-16)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-162 with condition USS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 2 |  |  |
| } |  |  |  |

Table 7.1.1.8.4.3.3-18: NonCellDefiningSSB (Table 7.1.1.8.4.3.3-14)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-84AAA |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| NonCellDefiningSSB-r17 ::= SEQUENCE { |  |  |  |  |
| absoluteFrequencySSB-r17 |  | Set to value of absoluteFrequencySSB for NCD-SSB in TS 38.508-1 [4] Table 6.2.3.1-8/9 |  |  |
| ssb-Periodicity-r17 |  | Not present | The periodicity of this NCD-SSB is same as SSB periodicity of the CD-SSB |  |
| ssb-TimeOffset-r17 |  | Not present | The time offset between the first burst of CD-SSB transmitted in the serving cell and the first burst of this NCD-SSB transmitted is zero |  |
| } |  |  |  |  |

Table 7.1.1.8.4.3.3-19: PUCCH-Config (Table 7.1.1.8.4.3.3-15)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-112 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-Config ::= SEQUENCE { |  |  |  |
| resourceToAddModList SEQUENCE (SIZE (1..maxNrofPUCCH-Resources)) OF PUCCH-Resource { | 16 entries |  |  |
| PUCCH-Resource[n] SEQUENCE { |  | entry 1- 16 n=1,2,…16 |  |
| secondHopPRB | PRB-Id | Table 7.1.1.8.4.3.3-20 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.4.3.3-20: PRB-Id (Table 7.1.1.8.4.3.3-19)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-109 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PRB-Id | Set to value of the L_RBs - nrofPRBs where L_RBs is LRB of initialDownlinkBWP-RedCap found in TS 38.508-1 [4] Table 6.2.3.1-8 or TS 38.508-1 [4] Table 6.2.3.1-9 and nrofPRBs is defined for the corresponding PUCCH-Resource (1 otherwise). |  |  |

##### 7.1.1.8.5	Separate BWP / IDLE / eRedCap

7.1.1.8.5.1	Test Purpose (TP)

(1)

with { UE supporting eRedCap and in NR RRC_IDLE state }

ensure that {

when { initialDownlinkBWP-RedCap is configured }

then { the UE shall monitor the PDCCH on the BWP configured by initialDownlinkBWP-RedCap }

}

(2)

with { UE supporting (e)RedCap and in NR RRC_IDLE state }

ensure that {

when { initialUplinkBWP-RedCap is configured and the UE needs to perform the Random Access procedure }

then { the UE shall perform the Random Access procedure by using the BWP configured by initialUplinkBWP-RedCap }

}

7.1.1.8.5.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.321, clauses 5.15.1, 6.2.1. Unless otherwise stated these are Rel-18 requirements.

[TS 38.321, clause 5.15.1]

Upon initiation of the Random Access procedure on a Serving Cell, after the selection of carrier for performing Random Access procedure as specified in clause 5.1.1, the MAC entity shall for the selected carrier of this Serving Cell:

1>	if PRACH occasions are not configured for the active UL BWP:

2>	if the UE is an (e)RedCap UE; and

2>	if initialUplinkBWP-RedCap is configured:

3>	switch the active UL BWP to BWP indicated by initialUplinkBWP-RedCap.

2>	else:

3>	switch the active UL BWP to BWP indicated by initialUplinkBWP.

2>	if the Serving Cell is an SpCell:

3>	if the UE is an (e)RedCap UE; and

3>	if initialDownlinkBWP-RedCap is configured:

4>	switch the active DL BWP to BWP indicated by initialDownlinkBWP-RedCap.

3>	else:

4>	switch the active DL BWP to BWP indicated by initialDownlinkBWP.

1>	else:

2>	if the Serving Cell is an SpCell:

3>	if the active DL BWP does not have the same bwp-Id as the active UL BWP:

4>	switch the active DL BWP to the DL BWP with the same bwp-Id as the active UL BWP.

1>	stop the bwp-InactivityTimer associated with the active DL BWP of this Serving Cell, if running.

1>	if the Serving Cell is SCell:

2>	stop the bwp-InactivityTimer associated with the active DL BWP of SpCell, if running.

1>	perform the Random Access procedure on the active DL BWP of SpCell and active UL BWP of this Serving Cell.

If the MAC entity receives a PDCCH for BWP switching of a Serving Cell, the MAC entity shall:

1>	if there is no ongoing Random Access procedure associated with this Serving Cell; or

1>	if the ongoing Random Access procedure associated with this Serving Cell is successfully completed upon reception of this PDCCH addressed to C-RNTI (as specified in clauses 5.1.4, 5.1.4a, and 5.1.5):

2>	cancel, if any, triggered consistent LBT failure for this Serving Cell;

2>	perform BWP switching to a BWP indicated by the PDCCH.

If the MAC entity receives a PDCCH for BWP switching for a Serving Cell(s) or a dormancy SCell group(s) while a Random Access procedure associated with that Serving Cell is ongoing in the MAC entity, it is up to UE implementation whether to switch BWP or ignore the PDCCH for BWP switching, except for the PDCCH reception for BWP switching addressed to the C-RNTI for successful Random Access procedure completion (as specified in clauses 5.1.4, 5.1.4a, and 5.1.5) in which case the UE shall perform BWP switching to a BWP indicated by the PDCCH. Upon reception of the PDCCH for BWP switching other than successful contention resolution, if the MAC entity decides to perform BWP switching, the MAC entity shall stop the ongoing Random Access procedure and initiate a Random Access procedure after performing the BWP switching; if the MAC decides to ignore the PDCCH for BWP switching, the MAC entity shall continue with the ongoing Random Access procedure on the Serving Cell.

Upon reception of RRC (re-)configuration for BWP switching for a Serving Cell while a Random Access procedure associated with that Serving Cell is ongoing in the MAC entity, the MAC entity shall stop the ongoing Random Access procedure and initiate a Random Access procedure after performing the BWP switching.

Upon reception of RRC (re-)configuration for BWP switching for a Serving Cell, cancel any triggered consistent LBT failure in this Serving Cell.

The MAC entity shall for each activated Serving Cell configured with bwp-InactivityTimer:

1>	if the defaultDownlinkBWP-Id is configured, and the active DL BWP is not the BWP indicated by the defaultDownlinkBWP-Id, and the active DL BWP is not the BWP indicated by the dormantBWP-Id if configured; or

1>	if the UE is neither a RedCap nor an eRedCap UE, and if the defaultDownlinkBWP-Id is not configured, and the active DL BWP is not the initialDownlinkBWP, and the active DL BWP is not the BWP indicated by the dormantBWP-Id if configured; or

1>	if the UE is an (e)RedCap UE, and if the defaultDownlinkBWP-Id is not configured, and initialDownlinkBWP-RedCap is not configured, and the active DL BWP is not the initialDownlinkBWP; or

1>	if the UE is an (e)RedCap UE, and if the defaultDownlinkBWP-Id is not configured, and initialDownlinkBWP-RedCap is configured, and the active DL BWP is not the initialDownlinkBWP-RedCap:

2>	if a PDCCH addressed to C-RNTI or CS-RNTI indicating downlink assignment or uplink grant is received on the active BWP; or

2>	if a PDCCH addressed to G-RNTI or G-CS-RNTI configured for multicast indicating downlink assignment is received on the active BWP; or

2>	if a PDCCH addressed to C-RNTI or CS-RNTI indicating downlink assignment or uplink grant is received for the active BWP; or

2>	if a MAC PDU is transmitted in a configured uplink grant and LBT failure indication is not received from lower layers; or

2>	if a MAC PDU is received in a configured downlink assignment for unicast or MBS multicast:

3>	if there is no ongoing Random Access procedure associated with this Serving Cell; or

3>	if the ongoing Random Access procedure associated with this Serving Cell is successfully completed upon reception of this PDCCH addressed to C-RNTI (as specified in clauses 5.1.4, 5.1.4a and 5.1.5):

4>	start or restart the bwp-InactivityTimer associated with the active DL BWP.

2>	if the bwp-InactivityTimer associated with the active DL BWP expires:

3>	if the defaultDownlinkBWP-Id is configured:

4>	perform BWP switching to a BWP indicated by the defaultDownlinkBWP-Id.

3>	else:

4>	if the UE is a (e)RedCap UE; and

4>	if initialDownlinkBWP-RedCap is configured:

5>	perform BWP switching to the initialDownlinkBWP-RedCap.

4>	else:

5>	perform BWP switching to the initialDownlinkBWP.

NOTE:	If a Random Access procedure is initiated on an SCell, both this SCell and the SpCell are associated with this Random Access procedure.

1>	if a PDCCH for BWP switching is received, and the MAC entity switches the active DL BWP:

2>	if the defaultDownlinkBWP-Id is configured, and the MAC entity switches to the DL BWP which is not indicated by the defaultDownlinkBWP-Id and is not indicated by the dormantBWP-Id if configured; or

2>	if the UE is neither a RedCap nor an eRedCap UE, and if the defaultDownlinkBWP-Id is not configured, and the MAC entity switches to the DL BWP which is not the initialDownlinkBWP and is not indicated by the dormantBWP-Id if configured; or

2>	if the UE is an (e)RedCap UE, and if the defaultDownlinkBWP-Id is not configured, and initialDownlinkBWP-RedCap is not configured, and the MAC entity switches to the DL BWP which is not the initialDownlinkBWP; or

2>	if the UE is an (e)RedCap UE, and if the defaultDownlinkBWP-Id is not configured, and initialDownlinkBWP-RedCap is configured, and the MAC entity switches to the DL BWP which is not the initialDownlinkBWP-RedCap:

3>	start or restart the bwp-InactivityTimer associated with the active DL BWP.

Upon initiation of the Random Access procedure, after selection of the carrier for performing Random Access procedure as specified in clause 5.1.1, if the UE is an (e)RedCap UE in RRC_IDLE or RRC_INACTIVE mode, the MAC entity shall:

1>	if initialUplinkBWP-RedCap is configured for the selected carrier:

2>	perform the Random Access procedure as specified in clause 5.1 by using the BWP configured by initialUplinkBWP-RedCap.

1>	else:

2>	perform the Random Access procedure as specified in clause 5.1 by using the BWP configured by initialUplinkBWP.

1>	if initialDownlinkBWP-RedCap is configured:

2>	if the Random Access procedure was initiated for SI request (as specified in TS 38.331 [5]) and the Random Access Resources for SI request have been explicitly provided by RRC, and if the selected carrier is SUL carrier:

3>	monitor the PDCCH on the BWP configured by initialDownlinkBWP.

2>	else:

3>	monitor the PDCCH on the BWP configured by initialDownlinkBWP-RedCap.

1>	else:

2>	monitor the PDCCH on the BWP configured by initialDownlinkBWP.

[TS 38.321, clause 6.2.1]

The MAC subheader consists of the following fields:

-	LCID: The Logical Channel ID field identifies the logical channel instance of the corresponding MAC SDU or the type of the corresponding MAC CE or padding as described in Tables 6.2.1-1 and 6.2.1-1c for the DL-SCH and Tables 6.2.1-2 and 6.2.1-2c for the UL-SCH. There is one LCID field per MAC subheader. The size of the LCID field is 6 bits. If the LCID field is set to 34 as in Table 6.2.1-1 or 6.2.1-2, one additional octet is present in the MAC subheader containing the eLCID field and follow the octet containing LCID field. If the LCID field is set to 33 as in Table 6.2.1-1 or 6.2.1-2, two additional octets are present in the MAC subheader containing the eLCID field and these two additional octets follow the octet containing LCID field;

NOTE 1:	For MBS broadcast, a logical channel is identified based on G-RNTI and LCID if the same LCID is allocated for logical channels corresponding to different G-RNTIs.

-	eLCID: The extended Logical Channel ID field identifies the logical channel instance of the corresponding MAC SDU or the type of the corresponding MAC CE as described in tables 6.2.1-1a, 6.2.1-1b, 6.2.1-2a and 6.2.1-2b for the DL-SCH and UL-SCH respectively. The size of the eLCID field is either 8 bits or 16 bits.

NOTE 2:	The extended Logical Channel ID space using two-octet eLCID and the relevant MAC subheader format is used, only when configured, on the NR backhaul links between IAB nodes or between IAB node and IAB Donor, or for multicast MTCHs.

-	L: The Length field indicates the length of the corresponding MAC SDU or variable-sized MAC CE in bytes. There is one L field per MAC subheader except for subheaders corresponding to fixed-sized MAC CEs, padding, and MAC SDUs containing UL CCCH. The size of the L field is indicated by the F field;

-	F: The Format field indicates the size of the Length field. There is one F field per MAC subheader except for subheaders corresponding to fixed-sized MAC CEs, padding, and MAC SDUs containing UL CCCH. The size of the F field is 1 bit. The value 0 indicates 8 bits of the Length field. The value 1 indicates 16 bits of the Length field;

-	LX: The LCID extension field indicates the use of extended LCID space. The size of the LX field is 1 bit. The LX field set to 1 indicates the use of Table 6.2.1-2c, otherwise R bit is present instead (i.e. set to 0);

-	R: Reserved bit, set to 0.

The MAC subheader is octet aligned.

…

Table 6.2.1-2c: Values of LCID for UL-SCH when the LX field is set to 1

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 | (216 + 320) | CCCH of size 48 bits for an eRedCap UE |
| 1 | (216 + 321) | CCCH of size 64 bits for an eRedCap UE |
| 2 | (216 + 322) | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK, except for an (e)RedCap UE |
| 3 | (216 + 323) | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK, except for an (e)RedCap UE |
| 4 | (216 + 324) | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK of a RedCap UE |
| 5 | (216 + 325) | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK of a RedCap UE |
| 6 | (216 + 326) | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK of an eRedCap UE |
| 7 | (216 + 327) | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK of an eRedCap UE |
| 8 to 63 | (216 + 328) to (216 + 383) | Reserved |
| NOTE 1: The MAC entity may use the code point corresponding to a given feature or feature combination in Table 6.2.1-2c only if network indicates support for the corresponding feature or feature combination. NOTE 2: CCCH of size 48 bits and CCCH of size 64 bits are referred to as CCCH and CCCH1, respectively, in TS 38.331 [5]. NOTE 3: For UE capable of PUCCH repetition of Msg4 HARQ-ACK, the MAC entity uses the code points corresponding to PUCCH repetition of Msg4 HARQ-ACK if numberOfMsg4HARQ-ACK-Repetitions is configured and rsrp-ThresholdMsg4HARQ-ACK is not configured, or if both are configured and the RSRP of the downlink pathloss reference is less than rsrp-ThresholdMsg4HARQ-ACK. |  |  |

7.1.1.8.5.3	Test description

7.1.1.8.5.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

UE:

-	None

Preamble:

-	The UE is in NR RRC_Idle mode (state 1N-A) on NR Cell 1 according to 38.508-1 [4] Table 4.4A.2-1.

7.1.1.8.5.3.2	Test procedure sequence

Table 7.1.1.8.5.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes the SIB1 of NR Cell 1 to configure initialDownlinkBWP-RedCap and initialUplinkBWP-RedCap. | - | - | - | - |
| 2 | The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. | <-- | PDCCH (DCI 1_0): Short Message | - | - |
| 3 | Wait for 2.1* modification period second for the UE to receive new system information. (Note 1) | - | - | - | - |
| 4 | The SS transmits a Paging message within the BWP configured by initialDownlinkBWP-RedCap. | <-- | NR RRC: Paging | - | - |
| 5 | Check: Does the UE transmit an RRCSetupRequest message by setting LCID field to 0 and LX field to 1? | --> | NR RRC: RRCSetupRequest | 1 | P |
| 6 | The SS transmits an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 7 | Check: Does the UE transmit HARQ ACK for step 6 on PUCCH configured by initialUplinkBWP-RedCap? | --> | HARQ ACK | 2 | P |
| 8-13a1 | Steps 4-9a1 of Generic procedure as specified in TS 38.508-1 [4] Table 4.9.4.2.2-1 are performed. | - | - | - | - |
| Note 1: The modification period, expressed in number of radio frames = modificationPeriodCoeff * defaultPagingCycle. |  |  |  |  |  |

7.1.1.8.3.3.3	Specific message contents

Table 7.1.1.8.5.3.3-1: SIB1 (step 1, Table 7.1.1.8.5.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| servingCellConfigCommon |  | ServingCellConfigCommonSIB | Table 7.1.1.8.5.3.3-2 |  |
| } |  |  |  |  |

Table 7.1.1.8.5.3.3-2: ServingCellConfigCommonSIB (Table 7.1.1.8.5.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| downlinkConfigCommon | DownlinkConfigCommonSIB | Table 7.1.1.8.5.3.3-3 |  |
| uplinkConfigCommon-v1700 ::= SEQUENCE { |  |  |  |
| initialUplinkBWP-RedCap-r17 | BWP-UplinkCommon | Table 7.1.1.8.5.3.3-7 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.5.3.3-3: DownlinkConfigCommonSIB (Table 7.1.1.8.5.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-53 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DownlinkConfigCommonSIB ::= SEQUENCE { |  |  |  |
| pei-Config-r17 | Not present |  |  |
| initialDownlinkBWP-RedCap-r17 | BWP-DownlinkCommon-RedCap | Table 7.1.1.8.5.3.3-4 |  |
| } |  |  |  |

Table 7.1.1.8.5.3.3-4: BWP-DownlinkCommon-RedCap (Table 7.1.1.8.5.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-10 with condition InitialBWP_SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkCommon ::= SEQUENCE { |  |  |  |
| pdcch-ConfigCommon CHOICE { |  |  |  |
| setup | PDCCH-ConfigCommon | Table 7.1.1.8.5.3.3-5 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.5.3.3-5: PDCCH-ConfigCommon (Table 7.1.1.8.5.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-96 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| commonSearchSpaceList SEQUENCE (SIZE (1..4)) OF SearchSpace { | 2 entries |  |  |
| SearchSpace[1] | SearchSpace with condition CSS as specified in TS 38.508-1 [4] Table 4.6.3-162 | entry 1 |  |
| SearchSpace[2] | SearchSpace | entry 2, Table 7.1.1.8.5.3.3-6 |  |
| } |  |  |  |
| searchSpaceOtherSystemInformation | Not present |  |  |
| pagingSearchSpace | 4 |  |  |
| } |  |  |  |

Table 7.1.1.8.5.3.3-6: SearchSpace (Table 7.1.1.8.5.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-162 with condition CSS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 4 |  |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl10 | 2 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.5.3.3-7: BWP-UplinkCommon (Table 7.1.1.8.5.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkCommon ::= SEQUENCE { |  |  |  |
| pucch-ConfigCommon CHOICE { |  |  |  |
| setup | PUCCH-ConfigCommon | Table 7.1.1.8.5.3.3-8 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.5.3.3-8: PUCCH-ConfigCommon (Table 7.1.1.8.5.3.3-7)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-113 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| pucch-ResourceCommon | Not present |  |  |
| nrofPRBs | Not present |  |  |
| intra-SlotFH-r17 | fromLowerEdge |  |  |
| pucch-ResourceCommon-RedCap-r17 | 1 |  |  |
| additionalPRBOffset-r17 | n2 |  |  |
| } |  |  |  |

##### 7.1.1.8.6	Separate BWP / RedCap-specific initial DL BWP without CORESET#0 / NCD-SSB / eRedCap

7.1.1.8.6.1	Test Purpose (TP)

(1)

with { UE supporting eRedCap and in NR RRC_IDLE state }

ensure that {

when { initialDownlinkBWP-RedCap does not contain the entire CORESET#0 }

then { UE uses this BWP for receiving DL messages during initial access and after initial access }

}

(2)

with { UE in NR RRC_CONNECTED and the active BWP is RedCap-specific initial downlink BWP and this BWP does not include CD-SSB and the entire CORESET#0 }

ensure that {

when { NCD-SSB is configured for an eRedCap UE in RRCSetup }

then { UE replies RRCSetupComplete }

}

(3)

with { UE in NR RRC_CONNECTED and the active BWP is RedCap-specific initial downlink BWP and this BWP does not include CD-SSB and the entire CORESET#0 }

ensure that {

when { SS sends RRCRelease to trigger UE move to RRC_IDLE }

then { UE shall monitor paging in the initial DL BWP that includes CORESET#0 }

}

7.1.1.8.6.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.300, clauses 16.13.5, and TS 38.331, clause 6.3.2. Unless otherwise stated these are Rel-18 requirements.

[TS 38.321, clause 5.15.1]

Upon initiation of the Random Access procedure, after selection of the carrier for performing Random Access procedure as specified in clause 5.1.1, if the UE is an (e)RedCap UE in RRC_IDLE or RRC_INACTIVE mode, the MAC entity shall:

1>	if initialUplinkBWP-RedCap is configured for the selected carrier:

2>	perform the Random Access procedure as specified in clause 5.1 by using the BWP configured by initialUplinkBWP-RedCap.

1>	else:

2>	perform the Random Access procedure as specified in clause 5.1 by using the BWP configured by initialUplinkBWP.

1>	if initialDownlinkBWP-RedCap is configured:

2>	if the Random Access procedure was initiated for SI request (as specified in TS 38.331 [5]) and the Random Access Resources for SI request have been explicitly provided by RRC, and if the selected carrier is SUL carrier:

3>	monitor the PDCCH on the BWP configured by initialDownlinkBWP.

2>	else:

3>	monitor the PDCCH on the BWP configured by initialDownlinkBWP-RedCap.

1>	else:

2>	monitor the PDCCH on the BWP configured by initialDownlinkBWP.

[TS 38.300, clause 16.13.5]

An (e)RedCap UE may be configured with multiple NCD-SSBs provided that each BWP is configured with at most one SSB. NCD-SSB may be configured for an (e)RedCap UE in RRC_CONNECTED to perform RLM, BFD, and RRM measurements and RA resource selection when the active BWP does not contain CD-SSB.

[TS 38.331, clause 6.3.2]

DownlinkConfigCommonSIB field descriptions

initialDownlinkBWP-RedCap If present, (e)RedCap UEs use this DL BWP instead of initialDownlinkBWP. If the locationAndBandwidth of this BWP contains the entire CORESET#0, the UE applies the locationAndBandwidth upon reception of this field (e.g. to determine the frequency position of signals described in relation to this locationAndBandwidth) but it keeps CORESET#0 until after reception of RRCSetup/RRCResume/RRCReestablishment. Otherwise, i.e., if the locationAndBandwidth of this BWP does not contain the entire CORESET#0, the UE uses this BWP for receiving DL messages during initial access (Msg2, MsgB, Msg4) and after initial access. If absent, (e)RedCap UEs use initialDownlinkBWP provided that it does not exceed the (e)RedCap UE maximum bandwidth (see also clause 5.2.2.4.2).

BWP-DownlinkDedicated field descriptions

nonCellDefiningSSB If configured, the UE operating in this BWP uses this SSB for the purposes for which it would otherwise have used the CD-SSB of the serving cell (e.g. obtaining sync, measurements, RLM, BFD, beam management). Furthermore, other parts of the BWP configuration that refer to an SSB (e.g. the "SSB" configured in the QCL-Info IE; the "ssb-Index" configured in the RadioLinkMonitoringRS; CFRA-SSB-Resource; PRACH-ResourceDedicatedBFR) refer implicitly to this NCD-SSB. The NCD-SSB has the same values for the properties (e.g., ssb-PositionsInBurst, PCI, ssb-PBCH-BlockPower) of the corresponding CD-SSB apart from the values of the properties configured in the NonCellDefiningSSB-r17 IE. In the MIB associated with this NCD-SSB, the systemFrameNumber field indicates the frame boundary and frame number of the NCD-SSB. The subCarrierSpacingCommon and dmrs-TypeA-Position field in the MIBs associated with CD-SSB and NCD-SSB in the same cell are configured with the same values, respectively.

PDCCH-ConfigCommon field descriptions

commonControlResourceSet An additional common control resource set which may be configured and used for any common or UE-specific search space. If the network configures this field, it uses a ControlResourceSetId other than 0 for this ControlResourceSet. The network configures the commonControlResourceSet in SIB1 so that it is contained in the bandwidth of CORESET#0. If the RedCap-specific initial downlink BWP does not contain the entire CORESET#0, the network configures the commonControlResourceSet in the RedCap-specific initial downlink BWP in SIB1 for (e)RedCap such that it does not have to be contained in the bandwidth of CORESET#0.

pagingSearchSpace ID of the search space for paging (see TS 38.213 [13], clause 10.1). If the field is absent, the UE does not receive paging in this BWP (see TS 38.213 [13], clause 10). This field is absent for the RedCap-specific initial downlink BWP, if it does not include CD-SSB and the entire CORESET#0. In that case, an (e)RedCap UE in RRC_INACTIVE while SDT procedure is ongoing and T319a is not running, if CG-SDT is selected and if extended CG-SDT periodicity is configured (i.e. cg-SDT-PeriodicityExt is configured), or an (e)RedCap UE in RRC_IDLE or RRC_INACTIVE while SDT procedure is not ongoing, shall monitor paging in the initial DL BWP that includes CORESET#0.

ra-SearchSpace ID of the Search space for random access procedure (see TS 38.213 [13], clause 10.1). If the field is absent, the UE does not receive RAR in this BWP. This field is mandatory present in the DL BWP(s) if the conditions described in TS 38.321 [3], clause 5.15 are met.

searchSpaceOtherSystemInformation ID of the Search space for other system information, i.e., SIB2 and beyond (see TS 38.213 [13], clause 10.1). If the field is absent, the UE does not receive other system information in this BWP. This field is absent for the RedCap-specific initial DL BWP, if it does not include CD-SSB and the entire CORESET#0. In that case, an (e)RedCap UE in RRC_IDLE or RRC_INACTIVE shall monitor PDCCH to receive other system information using searchSpaceOtherSystemInformation in the initial DL BWP that includes CD-SSB and the entire CORESET#0.

searchSpaceSIB1 ID of the search space for SIB1 message. In the initial DL BWP of the UE′s PCell, the network sets this field to 0. If the field is absent, the UE does not receive SIB1 in this BWP. (see TS 38.213 [13], clause 10). This field is absent for the RedCap-specific initial DL BWP, if it does not include CD-SSB and the entire CORESET#0. In that case, an (e)RedCap UE in RRC_IDLE or RRC_INACTIVE shall monitor PDCCH to receive SIB1 using searchSpaceSIB1 in the initial DL BWP that includes CD-SSB and the entire CORESET#0.

7.1.1.8.6.3	Test description

Same as 7.1.1.8.4.3 with exceptions below:

-	NR Cell 1 is configured to use Test frequency NRf1 as specified in TS 38.508-1 [4] Table 6.2.3.1-8 using the common highest mandatory UL and DL channel bandwidth and using the default subcarrier spacing specified in TS 38.508-1 [4] clause 6.2.3.1.

#### 7.1.1.9	MAC Reconfiguration and Reset

##### 7.1.1.9.1	MAC Reset

7.1.1.9.1.1	Test Purpose (TP)

(1)

Void

(2)

with { UE in RRC_CONNECTED state )

ensure that {

when{ UE MAC is reset, due to reconfiguration with sync on same cell }

then { UE considers the next transmission for each DL HARQ process as very first }

}

(3)

Void

(4)

with ( UE in RRC_CONNECTED state )

ensure that {

when{ UE MAC is reset, due to reconfiguration with sync on same cell }

then { UE flushes UL HARQ buffer }

}

(5)

with (UE in RRC_CONNECTED state )

ensure that {

when{ UE MAC is reset, due to reconfiguration with sync on same cell }

then { UE Considers the next transmission for each UL HARQ process as very first }

}

7.1.1.9.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.321, clauses 5.12 and TS 38.331 clause 5.3.5.5.2. Unless otherwise stated these are Rel-15 requirements.

[TS 38.321, clause 5.12]

If a reset of the MAC entity is requested by upper layers, the MAC entity shall:

1>	initialize Bj for each logical channel to zero;

1>	stop (if running) all timers;

1>	consider all timeAlignmentTimers as expired and perform the corresponding actions in subclause 5.2;

1>	set the NDIs for all uplink HARQ processes to the value 0;

1>	stop, if any, ongoing Random Access procedure;

1>	discard explicitly signalled contention-free Random Access Resources, if any;

1>	flush Msg3 buffer;

1>	cancel, if any, triggered Scheduling Request procedure;

1>	cancel, if any, triggered Buffer Status Reporting procedure;

1>	cancel, if any, triggered Recommended bit rate query procedure;

1>	cancel, if any, triggered Configured uplink grant confirmation;

1>	cancel, if any, triggered Power Headroom Reporting procedure;

1>	flush the soft buffers for all DL HARQ processes;

1>	for each DL HARQ process, consider the next received transmission for a TB as the very first transmission;

1>	release, if any, Temporary C-RNTI;

1>	reset BFI_COUNTER.

[TS 38.331, clause 5.3.5.5.2]

The UE shall perform the following actions to execute a reconfiguration with sync.

1>	if the AS security is not activated, perform the actions upon going to RRC_IDLE as specified in 5.3.11 with the release cause 'other' upon which the procedure ends;

1>	stop timer T310 for the corresponding SpCell, if running;

1>	start timer T304 for the corresponding SpCell with the timer value set to t304, as included in the reconfigurationWithSync;

1>	if the frequencyInfoDL is included:

2>	consider the target SpCell to be one on the SSB frequency indicated by the frequencyInfoDL with a physical cell identity indicated by the physCellId;

1>	else:

2>	consider the target SpCell to be one on the SSB frequency of the source SpCell with a physical cell identity indicated by the physCellId;

1>	start synchronising to the DL of the target SpCell;

1>	apply the specified BCCH configuration defined in 9.1.1.1;

1>	acquire the MIB, which is scheduled as specified in TS 38.213 [13];

NOTE 1:	The UE should perform the reconfiguration with sync as soon as possible following the reception of the RRC message triggering the reconfiguration with sync, which could be before confirming successful reception (HARQ and ARQ) of this message.

NOTE 2:	The UE may omit reading the MIB if the UE already has the required timing information, or the timing information is not needed for random access.

1>	reset the MAC entity of this cell group;

1>	consider the SCell(s) of this cell group, if configured, to be in deactivated state;

1>	apply the value of the newUE-Identity as the C-RNTI for this cell group;

1>	configure lower layers in accordance with the received spCellConfigCommon;

1>	configure lower layers in accordance with any additional fields, not covered in the previous, if included in the received reconfigurationWithSync.

7.1.1.9.1.3	Test description

7.1.1.9.1.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0.

Table 7.1.1.9.1.3.1-1: Void

7.1.1.9.1.3.2	Test procedure sequence

Table 7.1.1.9.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | The SS stops the default downlink retransmission. (Note 4) SS ignores scheduling requests and does not allocate any uplink grant until after step 3 | - | - | - | - |
| 1 | The SS transmits a MAC PDU containing one RLC SDU with P field set 0 on DRB | <-- | MAC PDU (1 RLC SDU of 40 bytes on DRB) | - | - |
| 2 | Void | - | - | - | - |
| 3 | The SS transmits NR RRCReconfiguration message with reconfigurationWithSync with the same SpCell. (Note 1) | <-- | RRCReconfiguration | - | - |
| 4 | The UE transmits an NR RRCReconfigurationComplete message. (Note 2) | --> | RRCReconfigurationComplete | - | - |
| - | EXCEPTION: UE may loop back the MAC PDU sent at step 1 depending upon UE implementation, SS waits for 1 sec for UE to loop back this data | - | - | - | - |
| 4Aa | The UE transmits a MAC PDU including one RLC SDU | --> | MAC PDU | - | - |
| 4a-5 | Void | - | - | - | - |
| 5A | The SS ignores scheduling requests and does not allocate any uplink grant. |  |  |  |  |
| 6 | The SS transmits a MAC PDU containing RLC SDU with P field set 0 on DRB. The HARQ Process and NDI on PDCCH is same as in step 1. The SS shall ensure that the HARQ process used at step 1 will not be used in between steps 3 and 5. | <-- | MAC PDU (1 RLC SDU of 40 bytes on DRB) | - | - |
| 7 | Check: Does the UE transmit a scheduling request? | --> | (SR) | 2 | P |
| - | Exception: The SS ignores following scheduling requests before step 9. | - | - | - | - |
| 8 | The SS allocates 1 UL Grant with size 384 bits and NDI indicates new transmission. (Note 5) | <-- | Uplink Grant | - | - |
| 9 | The UE transmits a MAC PDU including one RLC SDU with P field set 1. | --> | MAC PDU | - | - |
| 9A | The SS transmits a STATUS PDU on a different HARQ process than used in step 6. | <-- | STATUS PDU | - | - |
| 10-16 | Void | - | - | - | - |
| 16A | The SS ignores scheduling requests and does not allocate any uplink grant. | - | - | - | - |
| 17 | The SS transmits a MAC PDU containing RLC SDU with P field set 0 on DRB. | <-- | MAC PDU (1 RLC SDU of 40 bytes on DRB) | - | - |
| 18 | The UE transmits a scheduling request | --> | (SR) | - | - |
|  | Exception: The SS ignores following scheduling requests before step 20. | - | - | - | - |
| 19 | The SS allocates an UL Grant with size 384 bits for one HARQ process X, and NDI indicates new transmission. (Note 5) | <-- | Uplink Grant | - | - |
| 20 | The UE transmits a MAC PDU including one RLC SDU with P field set 1. | --> | MAC PDU | - | - |
| 20A | The SS transmits a STATUS PDU on a different HARQ process than used in step 17. | <-- | STATUS PDU | - | - |
| 21 | Void |  |  |  |  |
| 22 | The SS transmits NR RRCReconfiguration message with reconfigurationWithSync with the same SpCell. Note 1 | <-- | RRCReconfiguration | - | - |
| 23 | The UE transmits an NR RRCReconfigurationComplete message. Note 2 |  | RRCReconfigurationComplete | - | - |
| 24 | Void |  |  |  |  |
| 24A | The SS ignores scheduling requests and does not allocate any uplink grant. | - | - | - | - |
| 25 | The SS transmits a MAC PDU containing RLC SDU with P field set 0 on DRB. The HARQ Process and NDI on PDCCH is same as in step 17. The SS shall ensure that the HARQ process used at step 17 will not be used in between steps 22 and 23. | <-- | MAC PDU (1 RLC SDU of 40 bytes on DRB) | - | - |
| 26 | The UE transmits a scheduling request | --> | (SR) | - | - |
|  | Exception: The SS ignores following scheduling requests before step 28. | - | - | - | - |
| 27 | The SS allocates an UL Grant with size 384 bits corresponding to HARQ process X, with NDI not toggled compared to step 19, and NDI indicates new transmission. (Note 5) | <-- | Uplink Grant | - | - |
| 28 | Check: Does UE transmit a MAC PDU including one RLC SDU of 40 bytes on DRB and P field is set 1? | --> | MAC PDU | 4,5 | P |
| 29 | The SS transmits a STATUS PDU on a different HARQ process than used in step 25 | <-- | STATUS PDU | - | - |
| Note 1: for EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration. Note 2: for EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. Note 3: RLC re-establishment on DRB is used to make sure UE discard RLC PDU. Note 4: The SS stops the default downlink retransmission to avoid HARQ ACK for the retransmission of DRB data at step 1. Note 5: For pc_supportOfRedCap_r17=false AND pc_supportOfERedCap_r18= false, the UL grant of 384 bits (LRBs & IMCS as per 38.523-3[3] annex B) is chosen to allow the UE to transmit one PDU at a time (40 bytes RLC SDU + 3 bytes RLC Header + 2 bytes MAC Sub PDU header + 2 bytes for short BSR or padding). For pc_supportOfRedCap_r17=true OR pc_supportOfERedCap_r18=true, the UL grant of 384 bits is chosen to allow the UE to transmit one PDU at a time (40 bytes RLC SDU + 2 bytes RLC Header + 2 bytes MAC Sub PDU header + 3 bytes for short BSR and padding or only padding). |  |  |  |  |  |

7.1.1.9.1.3.3	Specific message contents

Table 7.1.1.9.1.3.3-0: SchedulingRequest-Config (Preamble)

| Derivation Path: 38.508-1 [4], Table 4.6.3-155 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| sr-TransMax | n64 |  |  |

Table 7.1.1.9.1.3.3-1: RRCReconfiguration for NR (steps 3 and 22 of Table 7.1.1.9.1.3.2-1)

| Derivation Path: 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig | According to Table 7.1.1.9.1.3.3-1A |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig | According to Table 7.1.1.9.1.3.3-1B |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.9.1.3.3-1A: RadioBearerConfig

| Derivation Path: 38.508-1 [4], Table 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | n entries | n is equal to the total number of DRBs established during preamble |  |
| DRB-ToAddMod[k=1..n] SEQUENCE { |  | entry (1..n) |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config | SDAP-Config | According to TS 38.508-1, Table 4.6.3-161 |  |
| } |  |  |  |
| drb-Identity | k | k=1..n |  |
| reestablishPDCP | true |  |  |
| recoverPDCP | Not present |  |  |
| pdcp-Config | PDCP-Config | According to Table 7.1.1.9.1.3.3-1C |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.9.1.3.3-1B: CellGroupConfig

| Derivation Path: 38.508-1 [4], Table 4.6.1-13 with condition PCell_change |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 2+n entries | n is equal to the total number of DRBs established during preamble |  |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions SRB1 and Re-establish_RLC | entry 1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions SRB2 and Re-establish_RLC | entry 2 |  |
| RLC-BearerConfig[k+2, k=1..n] | RLC-BearerConfig with conditions AM, DRBk and Re-establish_RLC | entry [k+2, k=1..n] |  |
| } |  |  |  |

Table 7.1.1.9.1.3.3-1C: PDCP-Config

| Derivation Path: 38.508-1 [4], Table 4.6.3-99 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-Config ::= SEQUENCE { |  |  |  |
| drb SEQUENCE { |  |  |  |
| statusReportRequired | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.9.1.3.3-2: RRCConnectionReconfiguration for EN-DC (step 3 and 22 of Table 7.1.1.9.1.3.2-1)

| Derivation Path: 36.508 Table 4.6.1-8 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |  |  |
| rrcConnectionReconfiguration-r8 ::= SEQUENCE { |  |  |  |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |  |  |  |
| nr-Config-r15 CHOICE { |  |  |  |  |  |  |
| nr-SecondaryCellGroupConfig-r15 | OCTET STRING including the RRCReconfiguration message and the IE secondaryCellGroup according to TS 38.508-1 [67], table 4.6.1-13 with condition EN-DC_HO |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| nr-RadioBearerConfig1-r15 | OCTET STRING including RadioBearerConfig according to TS 38.508-1 [67], table 4.6.3-132 with conditions EN-DC_DRB |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |

#### 7.1.1.10	Other Procedures

##### 7.1.1.10.1	DataInactivityTimer expiry

7.1.1.10.1.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state and dataInactivityTimer configured and running }

ensure that {

when { UE receives or transmits MAC SDU from DRB }

then { UE restarts the dataInactivityTimer }

}

(2)

with { UE in NR RRC_CONNECTED state and dataInactivityTimer configured and running }

ensure that {

when { UE detecting data inactivity on expiry of DataInactivityTimer }

then { UE enters RRC_IDLE state }

}

7.1.1.10.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 38.321 clause 5.19 and TS 38.331 clause 5.3.8.5.

[TS 38.321, clause 5.19]

The UE may be configured by RRC with a Data inactivity monitoring functionality, when in RRC_CONNECTED. RRC controls Data inactivity operation by configuring the timer dataInactivityTimer.

When dataInactivityTimer is configured, the UE shall:

1>	if any MAC entity receives a MAC SDU for DTCH logical channel, DCCH logical channel, or CCCH logical channel; or

1>	if any MAC entity transmits a MAC SDU for DTCH logical channel, or DCCH logical channel:

2>	start or restart dataInactivityTimer.

1>	if the dataInactivityTimer expires:

2>	indicate the expiry of the dataInactivityTimer to upper layers.

[TS 38.331 clause 5.3.8.5]

Upon receiving the expiry of DataInactivityTimer from lower layers while in RRC_CONNECTED, the UE shall:

1>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure'.

7.1.1.10.1.3	Test description

7.1.1.10.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

UE:

-	None.

Preamble:

-	The UE is in state 3N-A and Test Mode Activated according to 38.508-1 [4] Table 4.4A.2-1 with UE test loop mode B is established IP PDU delay set to 6 seconds.

7.1.1.10.1.3.2	Test procedure sequence

Table 7.1.1.10.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS waits (dataInactivityTimer/2 + 1) seconds | - | - | - | - |
| 2 | SS transmits a downlink assignment including the C-RNTI assigned to the UE | <-- | (PDCCH (C-RNTI)) | - | - |
| 3 | SS transmits in the indicated downlink assignment a RLC PDU in a MAC PDU. | <-- | MAC PDU | - | - |
| 4 | Void | - | - | - | - |
| 5 | Void | - | - | - | - |
| 6 | Check: Does the UE transmit a MAC PDU containing Loop backed PDU on expiry of IP PDU delay? | --> | MAC PDU (containing 1 MAC sub PDU containing RLC SDU) | 1 | P |
| 7-11 | Repeat steps 1-5 | - | - | - | - |
| 12 | Check: Does the UE transmit a MAC PDU containing Loop backed PDU? | --> | MAC PDU (containing 1 MAC sub PDU containing RLC SDU) | 1 | P |
| 13 | SS waits dataInactivityTimer seconds for the UE to enter RRC_IDLE. | - |  | - | - |
| 14 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.5.2.2-1 indicate that the UE is in RRC_IDLE? | - | - | 2 | - |

7.1.1.10.1.3.3	Specific Message Contents

Table 7.1.1.10.1.3.3-1: MAC-CellGroupConfig (preamble)

| Derivation path: 38.508-1[4], table 4.6.3-68 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| MAC-CellGroupConfig ::= SEQUENCE { |  |  |  |
| dataInactivityTimer | s10 |  |  |
| } |  |  |  |

Table 7.1.1.10.1.3.3-2: CLOSE UE TEST LOOP (Preamble)

| Derivation path: 36.508-1 [7] table 4.7A-3 condition UE test loop mode B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE test loop mode B LB setup |  |  |  |
| IP PDU delay | '0000 0110'B | 6 seconds |  |  |

##### 7.1.1.10.2	Recommended Bit Rate

7.1.1.10.2.1	Test Purpose (TP)

(1)

with { UE in RRC Connected state and MMTEL call established}

ensure that {

when { IF upper Layers requested to query the gNB for the recommended bit rate for a logical channel and for a direction and bitRateQueryProhibitTimer is not running }

then { UE transmits a Recommended Bit Rate Query MAC Control Element}

}

(2)

with(UE in RRC Connected state and MMTEL call established)

ensure that {

when{ IF upper Layers requested to query the gNB for the recommended bit rate for a logical channel and for a direction and bitRateQueryProhibitTimer is running}

then { UE does not transmits a Recommended Bit Rate Query MAC Control Element}

}

(3)

with ( UE in RRC Connected state and MMTEL call established)

ensure that {

when{ UE receives MAC PDU from the gNB for the recommended bit rate for a logical channel and for a direction }

then { UE sends an HARQ ACK }

}

7.1.1.10.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.321, clauses 5.18.10 and 6.1.20. Unless otherwise stated these are Rel-15 requirements.

[TS 38.321, clause 5.18.10]

The recommended bit rate procedure is used to provide the MAC entity with information about the bit rate which the gNB recommends. The bit rate is the recommended bit rate of the physical layer. Averaging window of default value 2000 ms will apply as specified in TS 26.114 [13].

The gNB may transmit the Recommended bit rate MAC CE to the MAC entity to indicate the recommended bit rate for the UE for a specific logical channel and a specific direction (either uplink or downlink). Upon reception of a Recommended bit rate MAC CE the MAC entity shall:

-	indicate to upper layers the recommended bit rate for the indicated logical channel and direction.

The MAC entity may request the gNB to indicate the recommended bit rate for a specific logical channel and a specific direction. If the MAC entity is requested by upper layers to query the gNB for the recommended bit rate for a logical channel and for a direction (i.e. for uplink or downlink), the MAC entity shall:

1>	if a Recommended bit rate query for this logical channel and this direction has not been triggered:

2>	trigger a Recommended bit rate query for this logical channel, direction, and desired bit rate.

If the MAC entity has UL resources allocated for new transmission the MAC entity shall:

1>	for each Recommended bit rate query that the Recommended Bit Rate procedure determines has been triggered and not cancelled:

2>	if bitRateQueryProhibitTimer for the logical channel and the direction of this Recommended bit rate query is configured, and it is not running; and

2>	if the MAC entity has UL resources allocated for new transmission and the allocated UL resources can accommodate a Recommended bit rate MAC CE plus its subheader as a result of LCP as defined in clause 5.4.3.1:

3>	instruct the Multiplexing and Assembly procedure to generate the Recommended bit rate MAC CE for the logical channel and the direction of this Recommended bit rate query;

3>	start the bitRateQueryProhibitTimer for the logical channel and the direction of this Recommended bit rate query;

3>	cancel this Recommended bit rate query.

[TS 38.321, clause 6.1.20]

The Recommended bit rate MAC CE is identified by a MAC subheader with LCID as specified in Tables 6.2.1-1 and 6.2.1-2 for bit rate recommendation message from the gNB to the UE and bit rate recommendation query message from the UE to the gNB, respectively. It has a fixed size and consists of two octets defined as follows (Figure 6.1.3.20-1):

-	LCID: This field indicates the identity of the logical channel for which the recommended bit rate or the recommended bit rate query is applicable. The length of the field is 6 bits;

-	Uplink/Downlink (UL/DL): This field indicates whether the recommended bit rate or the recommended bit rate query applies to uplink or downlink. The length of the field is 1 bit. The UL/DL field set to 0 indicates downlink. The UL/DL field set to 1 indicates uplink;

-	Bit Rate: This field indicates an index to Table 6.1.3.20-1. The length of the field is 6 bits. For bit rate recommendation the value indicates the recommended bit rate. For bit rate recommendation query the value indicates the desired bit rate;

-	R: reserved bit, set to 0.

Figure 6.1.3.20-1: Recommended bit rate MAC CE

Table 6.1.3.20-1: Values (kbit/s) for Bit Rate field

| Index | NR Recommended Bit Rate value [kbit/s] | Index | NR Recommended Bit Rate value [kbit/s] |
| --- | --- | --- | --- |
| 0 | Note 1 | 32 | 700 |
| 1 | 0 | 33 | 800 |
| 2 | 9 | 34 | 900 |
| 3 | 11 | 35 | 1000 |
| 4 | 13 | 36 | 1100 |
| 5 | 17 | 37 | 1200 |
| 6 | 21 | 38 | 1300 |
| 7 | 25 | 39 | 1400 |
| 8 | 29 | 40 | 1500 |
| 9 | 32 | 41 | 1750 |
| 10 | 36 | 42 | 2000 |
| 11 | 40 | 43 | 2250 |
| 12 | 48 | 44 | 2500 |
| 13 | 56 | 45 | 2750 |
| 14 | 72 | 46 | 3000 |
| 15 | 88 | 47 | 3500 |
| 16 | 104 | 48 | 4000 |
| 17 | 120 | 49 | 4500 |
| 18 | 140 | 50 | 5000 |
| 19 | 160 | 51 | 5500 |
| 20 | 180 | 52 | 6000 |
| 21 | 200 | 53 | 6500 |
| 22 | 220 | 54 | 7000 |
| 23 | 240 | 55 | 7500 |
| 24 | 260 | 56 | 8000 |
| 25 | 280 | 57 | Reserved |
| 26 | 300 | 58 | Reserved |
| 27 | 350 | 59 | Reserved |
| 28 | 400 | 60 | Reserved |
| 29 | 450 | 61 | Reserved |
| 30 | 500 | 62 | Reserved |
| 31 | 600 | 63 | Reserved |
| Note 1: For bit rate recommendation message this index is used for indicating that no new recommendation on bit rate is given. |  |  |  |

7.1.1.10.2.3	Test description

7.1.1.10.2.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used in NR cell.

UE:

-	None.

Preamble:

-	The UE is in 5GS state 1N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-1.

7.1.1.10.2.3.2	Test procedure sequence

Table 7.1.1.10.2.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The procedure in table 4.9.15.2.2-1 in TS 38.508-1 [4] is performed. The bitRateQueryProhibitTimer for the Logical channel of MMTEL QoS Flow is configured as s3 (3 seconds). | - | - | - | - |
| 2 | Trigger the UE to perform Recommended Bit Rate query for direction Downlink via AT (+CGBRRREQ) or MMI command. | - | - | - | - |
| 3 | Check: Does the UE transmit a MAC PDU containing Recommended bit rate MAC CE with Uplink/Downlink (UL/DL) set as 0? | -> | MAC PDU | 1 | P |
| 4 | Trigger the UE to perform Recommended Bit Rate query for direction Uplink via AT (+CGBRRREQ) or MMI command. | - | - | - | - |
| 5 | Check: Does the UE transmit a MAC PDU containing Recommended bit rate MAC CE with Uplink/Downlink (UL/DL) set as 1? | -> | MAC PDU | 1 | P |
| 6 | SS transmits a MAC PDU containing Recommended bit rate MAC CE with Uplink/Downlink (UL/DL) set as 0 and Bit Rate same as value received in step 5. | <- | MAC PDU | - | - |
| 7 | Trigger the UE to perform Recommended Bit Rate query for direction Up Link via AT (+CGBRRREQ) or MMI command. | - | - | - | - |
| 8 | Check: Does the UE transmit a MAC PDU containing Recommended bit rate MAC CE ? | -> | MAC PDU | 2 | P |
| 9 | While bitRateQueryProhibitTimer is running (3 seconds) in UE, trigger the UE to perform Recommended Bit Rate query for direction Up Link via AT (+CGBRRREQ) or MMI command. | - | - | - | - |
| 10 | Check: While bitRateQueryProhibitTimer is running, does the UE transmit a MAC PDU containing Recommended bit rate MAC CE ? | -> | MAC PDU | 2 | F |
| 11 | Check: After bitRateQueryProhibitTimer expires, does the UE transmit a MAC PDU containing Recommended bit rate MAC CE with Uplink/Downlink (UL/DL) set as 1? | -> | MAC PDU | 2 | P |
| 12 | SS transmits a MAC PDU containing Recommended bit rate MAC CE with Uplink/Downlink (UL/DL) set as 1 and Bit Rate same as value received in step 11. | <- | MAC PDU | - | - |
| 13 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 12? | --> | HARQ ACK | 3 | P |
| Note: The bitRateQueryProhibitTimer is configured only for UL direction as per asn.1 definition. |  |  |  |  |  |

7.1.1.10.2.3.3	Specific message contents

None

##### 7.1.1.10.3	NR CA / LBT failure on Scell / MAC CE indication

7.1.1.10.3.1	Test Purpose (TP)

(1)

with { UE in RRC_Connected state with SCell configured in uplink and Shared Spectrum }

ensure that {

when { UE detects consistent uplink LBT failures on the SCell }

then { UE transmits a MAC PDU containing LBT failure MAC Control Element on PCell }

}

7.1.1.10.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.321, clauses 5.21.1, 5.21.2 and 6.1.3.30. Unless otherwise stated these are Rel-16 requirements.

[TS 38.321, clause 5.21.1]

The lower layer may perform an LBT procedure, see TS 37.213 [18], according to which a transmission is not performed by lower layers if the channel is identified as being occupied. When lower layer performs an LBT procedure before a transmission and the transmission is not performed, an LBT failure indication is sent to the MAC entity from lower layers. Unless otherwise specified, when LBT procedure is performed for a transmission, actions as specified in this specification are performed regardless of if an LBT failure indication is received from lower layers. When LBT is not performed by the lower layers, LBT failure indication is not received from lower layers.

[TS 38.321, clause 5.21.2]

The MAC entity may be configured by RRC with a consistent LBT failure recovery procedure. Consistent LBT failure is detected per UL BWP by counting LBT failure indications, for all UL transmissions, from the lower layers to the MAC entity.

RRC configures the following parameters in the lbt-FailureRecoveryConfig:

-	lbt-FailureInstanceMaxCount for the consistent LBT failure detection;

-	lbt-FailureDetectionTimer for the consistent LBT failure detection;

The following UE variable is used for the consistent LBT failure detection procedure:

-	LBT_COUNTER (per Serving Cell): counter for LBT failure indication which is initially set to 0.

For each activated Serving Cell configured with lbt-FailureRecoveryConfig, the MAC entity shall:

1>	if LBT failure indication has been received from lower layers:

2>	start or restart the lbt-FailureDetectionTimer;

2>	increment LBT_COUNTER by 1;

2>	if LBT_COUNTER >= lbt-FailureInstanceMaxCount:

3>	trigger consistent LBT failure for the active UL BWP in this Serving Cell;

…

The MAC entity shall:

…

1>	else if consistent LBT failure has been triggered, and not cancelled, in at least one SCell:

2>	if UL-SCH resources are available for a new transmission in a Serving Cell for which consistent LBT failure has not been triggered and these UL-SCH resources can accommodate the LBT failure MAC CE plus its subheader as a result of logical channel prioritization:

3>	instruct the Multiplexing and Assembly procedure to generate the LBT failure MAC CE.

2>	else:

3>	trigger a Scheduling Request for LBT failure MAC CE.

1>	if a MAC PDU is transmitted and LBT failure indication is not received from lower layers and this PDU includes the LBT failure MAC CE:

2>	cancel all the triggered consistent LBT failure(s) in SCell(s) for which consistent LBT failure was indicated in the transmitted LBT failure MAC CE.

[TS 38.321, clause 6.1.3.30]

The LBT failure MAC CE of one octet is identified by a MAC subheader with LCID as specified in Table 6.2.1-2. It has a fixed size and consists of a single octet containing 8 C-fields as follows (Figure 6.1.3.30-1).

The LBT failure MAC CE of four octets is identified by a MAC subheader with LCID as specified in Table 6.2.1-2. It has a fixed size and consists of four octets containing 32 C-fields as follows (Figure 6.1.3.30-2).

A single octet format is used when the highest ServCellIndex of this MAC entity's Serving Cell for which LBT failure is detected is less than 8, otherwise four octets format is used.

-	Ci: If there is a Serving Cell configured for the MAC entity with ServCellIndex i as specified in TS 38.331 [5] and if consistent LBT failure have been triggered and not cancelled in this Serving Cell, the field is set to 1, otherwise the field is set to 0.

Figure 6.1.3.30-1: LBT failure MAC CE of one octet

Figure 6.1.3.30-2: LBT failure MAC CE of four octets

7.1.1.10.3.3	Test description

7.1.1.10.3.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the PCell.

-	NR Cell 10 is the SCell and configured to operate in Shared Spectrum.

-	System information combination NR-4 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used in NR cells.

UE:

-	None.

Preamble:

-	The UE is in 5GS state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3 and using the message condition UE TEST LOOP MODE A active.

7.1.1.10.3.3.2	Test procedure sequence

Table 7.1.1.10.3.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble.

Table 7.1.1.10.3.3.2-1: Cell configuration power level changes over time for conducted test environment

|  | Parameter | Unit | NR Cell 1 | NR Cell 10 |
| --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -88 |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -75 |
|  | Noc | dBm/SCS | - | -82 |

Table 7.1.1.10.3.3.2-2: Void

Table 7.1.1.10.3.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits an RRCReconfiguration message to configure NR SCell (NR Cell 10). | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | The SS transmits Activation MAC control element to activate NR SCell. | <-- | MAC PDU (SCell Activation/Deactivation MAC CE of one octet (C1=1)) | - | - |
| 4 | The SS does not allocate UL grants on NR PCell. | - | - | - | - |
| 5 | 200 ms after step 3, the SS indicates a new transmission on PDCCH of NR SCell and transmits a MAC PDU (containing an RLC PDU). | <-- | MAC PDU | - | - |
| 5A | The SS adjusts the SS/PBCH EPRE and Noc levels according to row "T1" in table 7.1.1.10.3.3.2-1. | - | - | - | - |
| 6 | The SS sends a periodic UL grant suitable for transmitting loop back PDU on NR SCell. | <-- | (UL Grant) | - | - |
| 7 | UE detects consistent LBT failures on NR SCell while attempting to loop back the data received at step 5. | - | - | - | - |
| 8 | The SS resumes normal UL grant allocation on NR PCell. | - | - | - | - |
| 9 | Check: Does the UE transmit a MAC PDU containing LBT failure MAC CE on NR PCell? | --> | MAC PDU (LBT failure MAC CE of one octet (C1=1)) | 1 | P |

7.1.1.10.3.3.3	Specific message contents

Table 7.1.1.10.3.3.3-1: RRCReconfiguration (step 1, Table 7.1.1.10.3.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.10.3.3.3-2: CellGroupConfig (Table 7.1.1.10.3.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SCellConfig { |  | 1 entry |  |  |
| SCellConfig[1] SEQUENCE { |  |  | entry 1 |  |
| sCellIndex |  | SCellIndex as per TS 38.508-1 [4] table 4.6.3-154 |  |  |
| sCellConfigCommon |  | ServingCellConfigCommon |  |  |
| sCellConfigDedicated |  | ServingCellConfig |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.10.3.3.3-3: ServingCellConfigCommon (Table 7.1.1.10.3.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-168 with conditions SCell_add and SharedSpectrum |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| physCellId | Physical Cell Identity of NR Cell 10 |  |  |
| uplinkConfigCommon | UplinkConfigCommon as per TS 38.508-1 [4] table 4.6.3-201 |  |  |
| } |  |  |  |

Table 7.1.1.10.3.3.3-4: ServingCellConfig (Table 7.1.1.10.3.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-167 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkDedicated with condition SharedSpectrum |  |  |
| } |  |  |  |
| } |  |  |  |

##### 7.1.1.10.4	NR NTN / UE specific TA report / UE specific Koffset

7.1.1.10.4.1	Test Purpose (TP)

(1)

with { UE in NR RRC_IDLE state on a cell which provides access by NR NTN and ta-Report-r17 is configured with value enabled in SIB19 }

ensure that {

when { UE initiates the Random Access procedure }

then { UE performs the Timing Advance reporting procedure }

}

(2)

with { UE in NR RRC_IDLE state on a cell which provides access by NR NTN and ta-Report-r17 is not configured in SIB19 }

ensure that {

when { offsetThresholdTA-r17 is received in rrcSetup message }

then { UE performs the Timing Advance reporting procedure }

}

(3)

with { UE in NR RRC_CONNECTED state and and cellSpecificKoffset-r17 is broadcasted in SIB19 }

ensure that {

when { Differential Koffset MAC CE is received }

then { UE applies received Differential Koffset and performs transmission on PUSCH }

}

7.1.1.10.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.321, clauses 5.4.8, 5.18.24, 6.1.3.56, 6.1.3.57 and TS 38.331, clauses 5.3.3.3, 6.3.2. Unless otherwise stated these are Rel-17 requirements.

[TS 38.321, clause 5.4.8]

The Timing Advance reporting procedure is used in a non-terrestrial network to provide the gNB with an estimate of the UE's Timing Advance value (i.e., TTA as defined in the UE's TA formula, see TS 38.211 [8] clause 4.3.1).

RRC controls Timing Advance reporting by configuring the following parameters:

-	offsetThresholdTA;

-	timingAdvanceSR.

A Timing Advance report (TAR) shall be triggered if any of the following events occur:

-	upon indication from upper layers to trigger a Timing Advance report;

-	upon configuration of offsetThresholdTA by upper layers, if the UE has not previously reported Timing Advance value to current Serving Cell;

-	if the variation between the current estimate of the Timing Advance value and the last reported Timing Advance value is equal to or larger than offsetThresholdTA, if configured.

The MAC entity shall:

1>	if the Timing Advance reporting procedure determines that at least one TAR has been triggered and not cancelled:

2>	if UL-SCH resources are available for a new transmission and the UL-SCH resources can accommodate the Timing Advance Report MAC CE plus its subheader as a result of logical channel prioritization:

3>	instruct the Multiplexing and Assembly procedure to generate the Timing Advance Report MAC CE as defined in clause 6.1.3.56.

2>	else

3>	if timingAdvanceSR is configured with value enabled:

4>	trigger a Scheduling Request.

NOTE:	UL-SCH resources are considered available if the MAC entity has been configured with, receives, or determines an uplink grant. If the MAC entity has determined at a given point in time that UL-SCH resources are available, this need not imply that UL-SCH resources are available for use at that point in time.

A MAC PDU shall contain at most one Timing Advance Report MAC CE, even when multiple events have triggered a Timing Advance report. The Timing Advance Report MAC CE shall be generated based on the latest available estimate of the UE's Timing Advance value prior to the MAC PDU assembly.

All triggered Timing Advance reports shall be cancelled when a MAC PDU is transmitted and this PDU includes a Timing Advance Report MAC CE.

[TS 38.321, clause 5.18.24]

The network may provide and update the Differential Koffset of a Serving Cell in a non-terrestrial network by sending the Differential Koffset MAC CE described in clause 6.1.3.57.

The MAC entity shall:

1>	if the MAC entity receives a Differential Koffset MAC CE on a Serving Cell:

2>	indicate to lower layers the information regarding the Differential Koffset MAC CE.

[TS 38.321, clause 6.1.3.56]

The Timing Advance Report MAC CE is identified by MAC subheader with LCID as specified in Table 6.2.1-2. It has a fixed size and consists of two octets defined as follows (Figure 6.1.3.56-1):

-	R: Reserved bit, set to 0;

-	Timing Advance: In FR1, the Timing Advance field indicates the least integer number of slots, using subcarrier spacing of 15 kHz, greater than or equal to the Timing Advance value (see TS 38.211 [8], clause 4.3.1). The length of the field is 14 bits.

Figure 6.1.3.56-1: Timing Advance Report MAC CE

[TS 38.321, clause 6.1.3.57]

The Differential Koffset MAC CE is identified by a MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a fixed size and consists of a single octet defined as follows (Figure 6.1.3.57-1):

-	R: Reserved bit, set to 0;

-	Differential Koffset: This field indicates the differential Koffset in the number of slots (see clause 4.2 in TS 38.213 [6]), using subcarrier spacing of 15 kHz. The length of the field is 6 bits.

Figure 6.1.3.57-1: Differential Koffset MAC CE

[TS 38.331, clause 5.3.3.3]

The UE shall set the contents of RRCSetupRequest message as follows:

…

1>	if ta-Report is configured with value enabled and the UE supports TA reporting:

2>	indicate TA report initiation to lower layers;

The UE shall submit the RRCSetupRequest message to lower layers for transmission.

[TS 38.331, clause 6.3.2]

offsetThresholdTA

Offset for TA reporting as specified in TS 38.321 [3]. Network only configures this parameter for MCG.

timingAdvanceSR

Used to configure whether a Timing Advance report may trigger a Scheduling Request as specified in TS 38.321 [3].

…

ta-Report

When this field is included in SIB19, it indicates reporting of timing advanced is enabled during Random Access due to RRC connection establishment or RRC connection resume, and during RRC connection reestablishment. When this field is included in ServingCellConfigCommon within dedicated signalling, it indicates TA reporting is enabled during Random Access due to reconfiguration with sync (see TS 38.321 [3], clause 5.4.8).

7.1.1.10.4.3	Test description

7.1.1.10.4.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 as specified in TS 38.508-1 [4] Table 4.4.2.1.

-	System information combination NR-29 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used.

UE:

-	The UE is in Automatic PLMN selection mode.

-	The pre-configured UE location is defined in TS 38.508-1 [4] Clause 4.5C.

Preamble

-	The UE is in state Switched OFF (state 0N-B) as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1.

7.1.1.10.4.3.2	Test procedure sequence

Table 7.1.1.10.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Power on the UE. | - | - | - | - |
| 2 | The UE transmits preamble on PRACH. | --> | PRACH Preamble | - | - |
| 3 | The SS transmits Random Access Response with RAPID corresponding to the transmitted preamble in step 2. | <-- | Random Access Response | - | - |
| 4 | Check: Does the UE transmit an RRCSetupRequest message with Timing Advance Report MAC CE included? | --> | MAC PDU (Timing Advance Report MAC CE, RRCSetupRequest) | 1 | P |
| 5-22 | Steps 3 to 20a1 of the registration procedure described in TS 38.508-1 [4] subclause 4.5.2.2-2 are performed. | - | - | - | - |
| 23 | The UE is switched off by executing generic procedure in Table 4.9.6.1-1 in TS 38.508-1 [4]. | - | - | - | - |
| 24 | SS broadcasts SIB19 with ta-Report-r17 not present. | <-- | RRC: SIB19 | - | - |
| 25 | Power on the UE. | - | - | - | - |
| 26 | The UE transmits preamble on PRACH. | --> | PRACH Preamble | - | - |
| 27 | The SS transmits Random Access Response with RAPID corresponding to the transmitted preamble in step 26. | <-- | Random Access Response | - | - |
| 28 | The UE transmits an RRCSetupRequest message with no Timing Advance Report MAC CE included. | --> | MAC PDU (RRCSetupRequest) | - | - |
| 29 | The SS transmits an RRCSetup message configuring offsetThresholdTA. | <-- | MAC PDU(RRCSetup) | - | - |
| 30 | Check: Does the UE transmit an RRCSetupComplete message with Timing Advance Report MAC CE included? | --> | MAC PDU (Timing Advance Report MAC CE, RRCSetupComplete) | 2 | P |
| 31-45 | Steps 5 to 19a1 of the registration procedure described in TS 38.508-1 [4] subclause 4.5.2.2-2 are performed. | - | - | - | - |
| 46 | The SS transmits a MAC PDU containing a Differential Koffset MAC CE with differential Koffset set to 20 ms (GSO scenario) or 1ms (NGSO scenario). | <-- | MAC PDU (Differential Koffset CE) | - | - |
| 47 | The SS transmits a UECapabilityEnquiry message | <-- | UECapabilityEnquiry | - | - |
| 48 | Check: Does the UE transmit a UECapabilityInformation message after applying differential Koffset of 20ms (GSO scenario) or 1ms (NGSO scenario) received at step 46? Note: Successful reception of this message at SS confirms that differential Koffset is applied at UE. | --> | UECapabilityInformation | 3 | P |

7.1.1.10.4.3.3	Specific message contents

Table 7.1.1.10.4.3.3-1: SIB19 (Preamble and all steps)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-18C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 | NTN-Config with condition GSO | Table 7.1.1.10.4.3.3-2 | GSO |
|  | NTN-Config with condition NGSO | Table 7.1.1.10.4.3.3-2 | NGSO |
| } |  |  |  |

Table 7.1.1.10.4.3.3-2: NTN-Config (Table 7.1.1.10.4.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-84C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NTN-Config-r17 ::= SEQUENCE { |  |  |  |
| ta-Report-r17 | enabled |  | Preamble |
|  | Not present |  | Step 24 |
| } |  |  |  |

Table 7.1.1.10.4.3.3-3: RRCSetup (Step 29, Table 7.1.1.10.4.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-21 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetup ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcSetup SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig with condition SRB1 | Table 7.1.1.10.4.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.10.4.3.3-4: CellGroupConfig (Table 7.1.1.10.4.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| mac-CellGroupConfig | MAC-CellGroupConfig |  |  |
| } |  |  |  |

Table 7.1.1.10.4.3.3-5: MAC-CellGroupConfig (Table 7.1.1.10.4.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-68 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MAC-CellGroupConfig ::= SEQUENCE { |  |  |  |
| TAR-Config-r17 ::= SEQUENCE { |  |  |  |
| offsetThresholdTA-r17 | ms15 |  |  |
| timingAdvanceSR-r17 | enabled |  |  |
| } |  |  |  |

##### 7.1.1.10.5	ATG / TA report / ta-ReportATG / offsetThresholdTA

7.1.1.10.5.1	Test Purpose (TP)

(1)

with { UE in NR RRC_IDLE state on a cell which provides ATG access and ta-ReportATG-r18 is configured with “enabled” in SIB22 }

ensure that {

when { UE initiates the Random Access procedure }

then { UE performs the Timing Advance reporting procedure }

}

(2)

with { UE in NR RRC_IDLE state on a cell which provides ATG access and ta-ReportATG-r18 is not configured in SIB22 }

ensure that {

when { UE receives an RRCSetup message and offsetThresholdTA-r18 is configured }

then { UE performs the Timing Advance reporting procedure }

}

7.1.1.10.5.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.300 clause 16.19.2.2, TS 38.321 clauses 5.4.8 and 6.1.3.56, TS 38.331 clause 6.3.2 . Unless otherwise stated these are Rel-18 requirements.

[TS 38.300 clause 16.19.2.2]

For the serving cell, the network broadcasts coarse gNB location information. The UE shall have valid GNSS position as well as gNB location before connecting to an ATG cell. To achieve synchronisation, before and during connection to an ATG cell, the UE shall compute the RTT between UE and the gNB based on the GNSS position and the gNB location parameters, and autonomously pre-compensate the TTA for the RTT between the UE and the gNB.

In connected mode, the UE shall be able to continuously update the Timing Advance.

The UE may be configured to report Timing Advance during Random Access procedures or in connected mode. For an RRC_CONNECTED UE, event-triggered reporting of the Timing Advance is supported.

[TS 38.321 clause 5.4.8]

The Timing Advance reporting procedure is used in a non-terrestrial network or an air to ground network to provide the gNB with an estimate of the UE's Timing Advance value (i.e., TTA as defined in the UE's TA formula, see TS 38.211 [8] clause 4.3.1).

RRC controls Timing Advance reporting by configuring the following parameters:

-	offsetThresholdTA;

-	timingAdvanceSR.

A Timing Advance report (TAR) shall be triggered if any of the following events occur:

-	upon indication from upper layers to trigger a Timing Advance report;

-	upon configuration of offsetThresholdTA by upper layers, if the UE has not previously reported Timing Advance value to current Serving Cell;

-	if the variation between the current estimate of the Timing Advance value and the last reported Timing Advance value is equal to or larger than offsetThresholdTA, if configured.

The MAC entity shall:

1>	if the Timing Advance reporting procedure determines that at least one TAR has been triggered and not cancelled:

2>	if UL-SCH resources are available for a new transmission and the UL-SCH resources can accommodate the Timing Advance Report MAC CE plus its subheader as a result of logical channel prioritization:

3>	instruct the Multiplexing and Assembly procedure to generate the Timing Advance Report MAC CE as defined in clause 6.1.3.56.

2>	else

3>	if timingAdvanceSR is configured with value enabled:

4>	trigger a Scheduling Request.

NOTE:	UL-SCH resources are considered available if the MAC entity has been configured with, receives, or determines an uplink grant. If the MAC entity has determined at a given point in time that UL-SCH resources are available, this need not imply that UL-SCH resources are available for use at that point in time.

A MAC PDU shall contain at most one Timing Advance Report MAC CE, even when multiple events have triggered a Timing Advance report. The Timing Advance Report MAC CE shall be generated based on the latest available estimate of the UE's Timing Advance value prior to the MAC PDU assembly.

All triggered Timing Advance reports shall be cancelled when a MAC PDU is transmitted and this PDU includes a Timing Advance Report MAC CE.

[TS 38.321 clause 6.1.3.56]

The Timing Advance Report MAC CE is identified by MAC subheader with LCID as specified in Table 6.2.1-2. It has a fixed size and consists of two octets defined as follows (Figure 6.1.3.56-1):

-	R: Reserved bit, set to 0;

-	Timing Advance: Except for ATG in FR1, the Timing Advance field indicates the least integer number of slots, using subcarrier spacing of 15 kHz, greater than or equal to the Timing Advance value (see TS 38.211 [8], clause 4.3.1).

For ATG in FR1, the Timing Advance field indicates the least integer number of symbols greater than or equal to the Timing Advance value (see TS 38.211 [8], clause 4.3.1). The symbol duration is based on the subcarrier spacing of the active BWP the UE is currently configured with. In this release of the specification, only 15 kHz and 30 kHz SCS are applicable and only values 1 … 56 are used.

The length of the field is 14 bits.

Figure 6.1.3.56-1: Timing Advance Report MAC CE

[TS 38.331 clause 6.3.2]

offsetThresholdTA

Offset for TA reporting as specified in TS 38.321 [3]. Network only configures this parameter for MCG. For offsetThresholdTA-r17, value ms0dot5 corresponds to 0.5 ms, value ms1 corresponds to 1 ms, and so on.

Network only configures offsetThresholdTA-r18 for ATG, which is in unit of symbols based on the subcarrier spacing of the BWP indicated by firstActiveDownlinkBWP-Id if received in the same RRC message, otherwise, it is based on the subcarrier spacing of the active DL BWP of the PCell when offsetThresholdTA-r18 is received. Once configured, the value of TA offset threshold does not change autonomously by the UE with BWP switching.

...

ta-ReportATG

When this field is included in SIB22, it indicates reporting of timing advanced is enabled during Random Access due to RRC connection establishment or RRC connection resume, and during RRC connection reestablishment. When this field is included in ServingCellConfigCommon within dedicated signalling, it indicates TA reporting is enabled during Random Access due to reconfiguration with sync (see TS 38.321 [3], clause 5.4.8).

7.1.1.10.5.3	Test description

7.1.1.10.5.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 as defined in TS 38.508-1 [4] Table 4.4.2-2 is used.

-	System information combination NR-34 as defined in TS 38.508-1 [4] clause 4.4.3.1 is used for NR cell.

UE:

-	The pre-configured UE location is defined in TS 38.508-1 [4] Clause 4.5D.

Preamble:

-	The UE is in Switched OFF (State 0N-B) as defined in TS 38.508-1 [4] Table 4.4A.2-0.

7.1.1.10.5.3.2	Test procedure sequence

Table 7.1.1.10.5.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Power on the UE. | - | - | - | - |
| 2 | The UE transmit a preamble on PRACH | --> | PRACH Preamble | - | - |
| 3 | The SS transmits Random Access Response with RAPID corresponding to the transmitted Preamble in step 2. | <-- | Random Access Response | - | - |
| 4 | Check: Does the UE transmit an MAC PDU with Timing Advance Report MAC CE included? | --> | MAC PDU (RRCSetupRequest, Timing Advance Report MAC CE) | 1 | P |
| 5-17 | Steps 3 to 15 of the procedure described in TS 38.508-1 [4] subclause 4.5.2.2-2 are performed. | - | - | - | - |
| 18 | The SS transmits an RRCRelease message. | <-- | RRCRelease | - | - |
| 19 | Power Off the UE. | - | - | - | - |
| 20 | The SS broadcasts the SIB22 with ta-Report-r18 set to not present. | <-- | RRC: SIB22 | - | - |
| 21 | Power on the UE. | - | - | - | - |
| 22 | The UE transmits a preamble on PRACH. | --> | PRACH Preamble | - | - |
| 23 | The SS transmits Random Access Response with RAPID corresponding to the transmitted Preamble in step 22. | <-- | Random Access Response | - | - |
| 24 | The UE transmits an RRCSetupRequest message with no Timing Advance Report MAC CE included. | --> | RRCSetupRequest | - | - |
| 25 | The SS transmits an RRCSetup message with offsetThresholdTA-r18 configured. | <-- | RRCSetup | - | - |
| 26 | Check: Does the UE transmit an MAC PDU with Timing Advance Report MAC CE included? | --> | MAC PDU (RRCSetupComplete, Timing Advance Report MAC CE) | 2 | P |
| 27-37 | Steps 5 to 15 of the procedure described in TS 38.508-1 [4] subclause 4.5.2.2-2 are performed. | - | - | - | - |
| 38-39 | Void | - | - | - | - |
| 40 | The SS transmits an RRCRelease message. | <-- | RRCRelease | - | - |

7.1.1.10.5.3.3	Specific message contents

Table 7.1.1.10.5.3.3-1: SIB22 (Preamble and all steps)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-21 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB22-r18 ::= SEQUENCE { |  |  |  |  |
| atg-Config-r18 |  | ATG-Config | Table 7.1.1.10.5.3.3-2 |  |
| } |  |  |  |  |

Table 7.1.1.10.5.3.3-2: ATG-Config (Table 7.1.1.10.5.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-5BA |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| ATG-Config-r18 ::= SEQUENCE { |  |  |  |  |
| atg-gNB-Location-r18 |  | ReferenceLocation | TS 38.508-1 [4], Table 4.6.3-138AA with condition RefLocation_1 |  |
| ta-ReportATG-r18 |  | enabled |  | preamble |
|  |  | Not present |  | Step 20 |
| } |  |  |  |  |

Table 7.1.1.10.5.3.3-3: RRCSetup (Step 25, Table 7.1.1.10.5.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-21 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCSetup ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcSetup SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig | Table 7.1.1.10.5.3.3-4 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.10.5.3.3-4: CellGroupConfig (Table 7.1.1.10.5.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |  |
| mac-CellGroupConfig |  | MAC-CellGroupConfig | Table 7.1.1.10.5.3.3-5 |  |
| } |  |  |  |  |

Table 7.1.1.10.5.3.3-5: MAC-CellGroupConfig (Table 7.1.1.10.5.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-68 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MAC-CellGroupConfig ::= SEQUENCE { |  |  |  |  |
| tar-Config-r18 SEQUENCE { |  |  |  |  |
| offsetThresholdTA-r18 |  | 5 |  |  |
| timingAdvanceSR-r18 |  | enabled |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

#### 7.1.1.11	NR Dual Connectivity

##### 7.1.1.11.1	DC power headroom reporting / PSCell activation and DL pathloss change reporting

7.1.1.11.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state on Pcell and PSCell is added }

ensure that {

when { phr is configured }

then { UE transmits a Power Headroom Report for the PCell and PSCell }

}

(2)

with { UE in RRC_CONNECTED state with PSCell and with Power headroom reporting for phr-Tx-PowerFactorChange }

ensure that {

when { the DL Pathloss has changed more than phr-Tx-PowerFactorChange dB and phr-ProhibitTimer is running }

then { UE does not transmit a MAC PDU containing Power Headroom MAC Control Element }

}

(3)

with { UE in RRC_CONNECTED state with PSCell and with Power headroom reporting for phr-Tx-PowerFactorChange }

ensure that {

when { the phr-ProhibitTimer expires and power headroom report is triggered due to DL Pathloss change  }

then { UE transmits a MAC PDU containing Power Headroom MAC Control Element for the Pcell and PSCell }

}

7.1.1.11.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 38.321 clause 5.4.6. Unless otherwise stated these are Rel-15 requirements.

[TS 38.321, clause 5.4.6]

A Power Headroom Report (PHR) shall be triggered if any of the following events occur:

-	phr-ProhibitTimer expires or has expired and the path loss has changed more than phr-Tx-PowerFactorChange dB for at least one activated Serving Cell of any MAC entity which is used as a pathloss reference since the last transmission of a PHR in this MAC entity when the MAC entity has UL resources for new transmission;

NOTE 1:	The path loss variation for one cell assessed above is between the pathloss measured at present time on the current pathloss reference and the pathloss measured at the transmission time of the last transmission of PHR on the pathloss reference in use at that time, irrespective of whether the pathloss reference has changed in between.

-	phr-PeriodicTimer expires;

-	upon configuration or reconfiguration of the power headroom reporting functionality by upper layers, which is not used to disable the function;

-	activation of an SCell of any MAC entity with configured uplink;

-	addition of the PSCell (i.e. PSCell is newly added or changed);

-	phr-ProhibitTimer expires or has expired, when the MAC entity has UL resources for new transmission, and the following is true for any of the activated Serving Cells of any MAC entity with configured uplink:

-	there are UL resources allocated for transmission or there is a PUCCH transmission on this cell, and the required power backoff due to power management (as allowed by P-MPRc as specified in TS 38.101-1 [14], TS 38.101-2 [15], and TS 38.101-3 [16]) for this cell has changed more than phr-Tx-PowerFactorChange dB since the last transmission of a PHR when the MAC entity had UL resources allocated for transmission or PUCCH transmission on this cell.

NOTE 2:	The MAC entity should avoid triggering a PHR when the required power backoff due to power management decreases only temporarily (e.g. for up to a few tens of milliseconds) and it should avoid reflecting such temporary decrease in the values of PCMAX,f,c/PH when a PHR is triggered by other triggering conditions.

If the MAC entity has UL resources allocated for a new transmission the MAC entity shall:

1>	if it is the first UL resource allocated for a new transmission since the last MAC reset:

2>	start phr-PeriodicTimer;

1>	if the Power Headroom reporting procedure determines that at least one PHR has been triggered and not cancelled; and

1>	if the allocated UL resources can accommodate the MAC CE for PHR which the MAC entity is configured to transmit, plus its subheader, as a result of LCP as defined in clause 5.4.3.1:

2>	if multiplePHR with value true is configured:

3>	for each activated Serving Cell with configured uplink associated with any MAC entity:

4>	obtain the value of the Type 1 or Type 3 power headroom for the corresponding uplink carrier as specified in clause 7.7 of TS 38.213 [6] for NR Serving Cell and clause 5.1.1.2 of TS 36.213 [17] for E-UTRA Serving Cell;

4>	if this MAC entity has UL resources allocated for transmission on this Serving Cell; or

4>	if the other MAC entity, if configured, has UL resources allocated for transmission on this Serving Cell and phr-ModeOtherCG is set to real by upper layers:

5>	obtain the value for the corresponding PCMAX,f,c field from the physical layer.

3>	if phr-Type2OtherCell with value true is configured:

4>	if the other MAC entity is E-UTRA MAC entity:

5>	obtain the value of the Type 2 power headroom for the SpCell of the other MAC entity (i.e. E-UTRA MAC entity);

5>	if phr-ModeOtherCG is set to real by upper layers:

6>	obtain the value for the corresponding PCMAX,f,c field for the SpCell of the other MAC entity (i.e. E-UTRA MAC entity) from the physical layer.

3>	instruct the Multiplexing and Assembly procedure to generate and transmit the Multiple Entry PHR MAC CE as defined in clause 6.1.3.9 based on the values reported by the physical layer.

2>	else (i.e. Single Entry PHR format is used):

3>	obtain the value of the Type 1 power headroom from the physical layer for the corresponding uplink carrier of the PCell;

3>	obtain the value for the corresponding PCMAX,f,c field from the physical layer;

3>	instruct the Multiplexing and Assembly procedure to generate and transmit the Single Entry PHR MAC CE as defined in clause 6.1.3.8 based on the values reported by the physical layer.

2>	start or restart phr-PeriodicTimer;

2>	start or restart phr-ProhibitTimer;

2>	cancel all triggered PHR(s).

7.1.1.11.1.3	Test description

7.1.1.11.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the PCell and NR Cell 10 is the PSCell.

-	System information combination NR-4 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used in all cells.

UE:

-	None.

Preamble:

-	The UE is in state NR RRC_CONNECTED using generic procedure parameter Connectivity (NR-DC), Test Mode (On) associated with UE test loop mode A configured on NR Cell 1 according to TS 38.508-1 [4], clause 4.5.4.

7.1.1.11.1.3.2	Test procedure sequence

Table 7.1.1.11.1.3.2-0: Cell configuration power level changes over time for Conducted test environment

|  | Parameter | Unit | NR Cell 1 | NR Cell 10 | Remarks |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -82 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -89 | -82 |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -82 |  |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -89 |  |
| T4 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -82 |  |

Table 7.1.1.11.1.3.2-0A: Cell configuration power level changes over time for OTA test environment

|  | Parameter | Unit | NR Cell 1 | NR Cell 10 | Remarks |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -82 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | n/a | n/a |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | n/a | n/a |  |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -91 |  |
| T4 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -82 |  |

Table 7.1.1.11.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits UL grant on PCell and PSCell to the UE at every 10ms in PDCCH occasion. | <-- | - | - | - |
| 2 | SS transmits NR RRCReconfiguration message to configure to specific Power Headroom parameters for NR Cell. | <-- | (RRCReconfiguration) | - | - |
| 3 | Check: Does the UE transmit a MAC PDU containing Multiple-Entry PHR MAC CE on PCell? (Note 1) | --> | MAC PDU | 1 | P |
| 3A | Check: Does the UE transmit a MAC PDU containing Multiple-Entry PHR MAC CE on PSCell? (Note 1) | --> | MAC PDU | 1 | P |
| 4 | The UE transmits an NR RRCReconfigurationComplete message including nr-SCG-Response. (Note 1) | --> | (RRCReconfigurationComplete) | - | - |
| 5 | Void | - | - | - | - |
| - | EXCEPTION: Steps 6 to 12 shall be executed depending on PSCell Configuration. (Note 3) | - | - | - | - |
| 6 | IF PSCell is configured as FR1 THEN Reduce SS power level for NR PCell so as to cause a DL_Pathloss change at UE by 5dB, row T1 of Table 7.1.1.11.1.3.2-0. | - | - | - | - |
| 7 | Check: For 80% of prohibitPHR-Timer since step 3, does the UE transmit a MAC PDU containing Multiple-Entry PHR MAC CE on PCell? | --> | MAC PDU | 2 | F |
| 8 | Check: After prohibitPHR-Timer after step 3, does the UE transmit a MAC PDU containing Multiple-Entry PHR MAC CE on PCell? | --> | MAC PDU | 3 | P |
| 9 | Increase SS power level for NR PCell so as to cause a DL_Pathloss change at UE by 5dB, row T2 of Table 7.1.1.11.1.3.2-0. | - | - | - | - |
| 10 | Check: For 80% of prohibitPHR-Timer since step 8, does the UE transmit a MAC PDU containing Power Headroom MAC Control Element on PCell? | --> | MAC PDU | 2 | F |
| 11 | Check: After prohibitPHR-Timer after step 8, does the UE transmit a MAC PDU containing Power Headroom MAC Control Element on PCell? | --> | MAC PDU | 3 | P |
| 12 | Void | - | - | - | - |
| 13 | Reduce SS power level for NR PSCell so as to cause a DL_Pathloss change at UE by 5dB, row T3 of Table 7.1.1.11.1.3.2-0/0A. | - | - | - | - |
| 14 | IF PSCell is configured as FR2 THEN Check: For 80% of prohibitPHR-Timer since step 3A, does the UE transmit a MAC PDU containing Multiple-Entry PHR MAC CE? | --> | MAC PDU | 2 | F |
| 15 | Check: Does the UE transmit a MAC PDU containing Multiple-Entry PHR MAC CE on PSCell? | --> | MAC PDU | 3 | P |
| 16 | Increase SS power level for NR PSCell so as to cause a DL_Pathloss change at UE by 5dB, row T4 of Table 7.1.1.11.1.3.2-0/0A. | - | - | - | - |
| 17 | Check: For 80% of prohibitPHR-Timer since step 15, does the UE transmit a MAC PDU containing Power Headroom MAC Control Element on PSCell? | --> | MAC PDU | 2 | F |
| 18 | Check: After prohibitPHR-Timer after step 15, does the UE transmit a MAC PDU containing Power Headroom MAC Control Element on PSCell? | --> | MAC PDU | 3 | P |
| 19 | The SS transmits an NR RRCReconfiguration message to disable Power Headroom reporting. | <-- | (RRCReconfiguration) | - | - |
| 20 | The UE transmits an NR RRCReconfigurationComplete message to confirm the disabling of Power Headroom parameters. | --> | (RRCReconfigurationComplete) | - | - |
| Note 1: Steps 3 and 4 can happen in any order. Note 2: Void. Note 3: Steps 6 to 12 are excluded when executed with FR1+FR2 band combination due to limitation in FR1 OTA requirements specified in 38.508-1 [4] clause 6.2.2.2.3. phr-Tx-PowerFactorChange for PCell is not tested due to this limitation |  |  |  |  |  |

7.1.1.11.1.3.3	Specific Message Contents

Table 7.1.1.11.1.3.3-1: RRCReconfiguration (step 2, Table 7.1.1.11.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig-phr | Table 7.1.1.11.1.3.3-2 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| mrdc-SecondaryCellGroupConfig CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| mrdc-ReleaseAndAdd |  | Not present |  |  |
| mrdc-SecondaryCellGroup CHOICE { |  |  |  |  |
| nr-SCG |  | RRCReconfiguration-SCG-phr | OCTET STRING (CONTAINING RRCReconfiguration) |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.11.1.3.3-1A: RRCReconfiguration-SCG-phr (Table 7.1.1.11.1.3.3-1)

| Derivation Path: TS 38. 508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | Not present |  |  |
| secondaryCellGroup |  | CellGroupConfig-phr | Table 7.1.1.11.1.3.3-2 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.11.1.3.3-2: CellGroupConfig-phr (Tables 7.1.1.11.1.3.3-1 and 7.1.1.11.1.3. 3-1 A)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| mac-CellGroupConfig SEQUENCE { |  |  |  |
| phr-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| phr-PeriodicTimer | infinity |  |  |
| phr-ProhibitTimer | sf500 |  |  |
| phr-Tx-PowerFactorChange | dB3 |  |  |
| multiplePHR | true |  |  |
| dummy | false |  |  |
| phr-Type2OtherCell | false |  |  |
| phr-ModeOtherCG | real |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 7.1.1.12	UE power saving

##### 7.1.1.12.1	Void

##### 7.1.1.12.2	Void

##### 7.1.1.12.3	DRX adaptation / UE wakeup indication

7.1.1.12.3.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and long DRX is configured and [(SFN * 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset and DCP is configured }

ensure that {

when { a DCP indication with the value of wake-up indication 1 associated with the current DRX cycle has been received }

then { UE starts the drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe and monitors the PDCCH }

}

(2)

with { UE in RRC_CONNECTED state and long DRX is configured and [(SFN * 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset and DCP is configured and ps-wakeup is configured with value true }

ensure that {

when { DCP indication associated with this cycle has not been received }

then { UE starts the drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe and monitors the PDCCH for OnDurationTimer PDCCH-Occasions }

}

(3)

with { UE in RRC_CONNECTED state long DRX is configured and [(SFN * 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset and DCP is configured }

ensure that {

when { all DCP occasions in time domain occurred in DRX active time }

then { UE does not monitor PDCCH for the detection of DCI format 2_6 and start the drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe and monitors the PDCCH }

}

(4)

with { UE in RRC_CONNECTED state long DRX is configured and DCP is configured }

ensure that {

when { all DCP occasions in time domain occurred during measurement gap }

then { UE does not monitor PDCCH for the detection of DCI format 2_6 and start the drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe and monitors the PDCCH }

}

(5)

with { UE in RRC_CONNECTED state and long DRX is configured and [(SFN * 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset and DCP is configured }

ensure that {

when { a DCP indication with the value of wake-up indication 0 associated with the current DRX cycle has been received }

then { UE does not start the drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe and skips monitoring the PDCCH }

}

7.1.1.12.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.321, clause 5.7, TS 38.213, clause 10.3 and 7.3.1.3.7. Unless otherwise stated these are Rel-16 requirements.

[TS 38.321, clause 5.7]

The MAC entity may be configured by RRC with a DRX functionality that controls the UE's PDCCH monitoring activity for the MAC entity's C-RNTI, CI-RNTI, CS-RNTI, INT-RNTI, SFI-RNTI, SP-CSI-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, TPC-SRS-RNTI, and AI-RNTI. When using DRX operation, the MAC entity shall also monitor PDCCH according to requirements found in other clauses of this specification. When in RRC_CONNECTED, if DRX is configured, for all the activated Serving Cells, the MAC entity may monitor the PDCCH discontinuously using the DRX operation specified in this clause; otherwise the MAC entity shall monitor the PDCCH as specified in TS 38.213 [6].

NOTE 1:	If Sidelink resource allocation mode 1 is configured by RRC, a DRX functionality is not configured.

RRC controls DRX operation by configuring the following parameters:

-	drx-onDurationTimer: the duration at the beginning of a DRX Cycle;

-	drx-SlotOffset: the delay before starting the drx-onDurationTimer;

-	drx-InactivityTimer: the duration after the PDCCH occasion in which a PDCCH indicates a new UL or DL transmission for the MAC entity;

-	drx-RetransmissionTimerDL (per DL HARQ process except for the broadcast process): the maximum duration until a DL retransmission is received;

-	drx-RetransmissionTimerUL (per UL HARQ process): the maximum duration until a grant for UL retransmission is received;

-	drx-LongCycleStartOffset: the Long DRX cycle and drx-StartOffset which defines the subframe where the Long and Short DRX Cycle starts;

-	drx-ShortCycle (optional): the Short DRX cycle;

-	drx-ShortCycleTimer (optional): the duration the UE shall follow the Short DRX cycle;

-	drx-HARQ-RTT-TimerDL (per DL HARQ process except for the broadcast process): the minimum duration before a DL assignment for HARQ retransmission is expected by the MAC entity;

-	drx-HARQ-RTT-TimerUL (per UL HARQ process): the minimum duration before a UL HARQ retransmission grant is expected by the MAC entity;

-	ps-Wakeup (optional): the configuration to start associated drx-onDurationTimer in case DCP is monitored but not detected;

-	ps-TransmitOtherPeriodicCSI (optional): the configuration to report periodic CSI that is not L1-RSRP on PUCCH during the time duration indicated by drx-onDurationTimer in case DCP is configured but associated drx-onDurationTimer is not started;

-	ps-TransmitPeriodicL1-RSRP (optional): the configuration to transmit periodic CSI that is L1-RSRP on PUCCH during the time duration indicated by drx-onDurationTimer in case DCP is configured but associated drx-onDurationTimer is not started.

Serving Cells may be configured by RRC in two groups. When RRC does not configure a secondary DRX group, there is only one DRX group. When two DRX groups are configured each group of Serving Cells, which is called a DRX group, is configured by RRC with its own set of parameters: drx-onDurationTimer, drx-InactivityTimer. When two DRX groups are configured, the two groups share the following parameter values: drx-SlotOffset, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-LongCycleStartOffset, drx-ShortCycle (optional), drx-ShortCycleTimer (optional), drx-HARQ-RTT-TimerDL, and drx-HARQ-RTT-TimerUL.

When a DRX cycle is configured, the Active Time for Serving Cells in a DRX group includes the time while:

-	drx-onDurationTimer or drx-InactivityTimer configured for the DRX group is running; or

-	drx-RetransmissionTimerDL or drx-RetransmissionTimerUL is running on any Serving Cell in the DRX group; or

-	ra-ContentionResolutionTimer (as described in clause 5.1.5) or msgB-ResponseWindow (as described in clause 5.1.4a) is running; or

-	a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4); or

-	a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clauses 5.1.4 and 5.1.4a).

When DRX is configured, the MAC entity shall:

1>	if the Long DRX Cycle is used, and [(SFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset:

2>	if DCP monitoring is configured for the active DL BWP as specified in TS 38.213 [6], clause 10.3:

3>	if DCP indication associated with the current DRX Cycle received from lower layer indicated to start drx-onDurationTimer, as specified in TS 38.213 [6]; or

3>	if all DCP occasion(s) in time domain, as specified in TS 38.213 [6], associated with the current DRX Cycle occurred in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to start of the last DCP occasion, or within BWP switching interruption length, or during a measurement gap; or

3>	if ps-Wakeup is configured with value true and DCP indication associated with the current DRX Cycle has not been received from lower layers:

4>	start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe.

2>	else:

3>	start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe.

NOTE 2:	In case of unaligned SFN across carriers in a cell group, the SFN of the SpCell is used to calculate the DRX duration.

1>	if the DRX group is in Active Time:

2>	monitor the PDCCH on the Serving Cells in this DRX group as specified in TS 38.213 [6];

2>	if the PDCCH indicates a DL transmission:

3>	start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback;

NOTE 3:	When HARQ feedback is postponed by PDSCH-to-HARQ_feedback timing indicating a non-numerical k1 value, as specified in TS 38.213 [6], the corresponding transmission opportunity to send the DL HARQ feedback is indicated in a later PDCCH requesting the HARQ-ACK feedback.

3>	stop the drx-RetransmissionTimerDL for the corresponding HARQ process.

3>	if the PDSCH-to-HARQ_feedback timing indicate a non-numerical k1 value as specified in TS 38.213 [6]:

4>	start the drx-RetransmissionTimerDL in the first symbol after the PDSCH transmission for the corresponding HARQ process.

2>	if the PDCCH indicates a UL transmission:

3>	start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first repetition of the corresponding PUSCH transmission;

3>	stop the drx-RetransmissionTimerUL for the corresponding HARQ process.

2>	if the PDCCH indicates a new transmission (DL or UL) on a Serving Cell in this DRX group:

3>	start or restart drx-InactivityTimer for this DRX group in the first symbol after the end of the PDCCH reception.

1>	if DCP monitoring is configured for the active DL BWP as specified in TS 38.213 [6], clause 10.3; and

1>	if the current symbol n occurs within drx-onDurationTimer duration; and

1>	if drx-onDurationTimer associated with the current DRX cycle is not started as specified in this clause:

2>	if the MAC entity would not be in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause:

3>	not transmit periodic SRS and semi-persistent SRS defined in TS 38.214 [7];

3>	not report semi-persistent CSI configured on PUSCH;

3>	if ps-TransmitPeriodicL1-RSRP is not configured with value true:

4>	not report periodic CSI that is L1-RSRP on PUCCH.

3>	if ps-TransmitOtherPeriodicCSI is not configured with value true:

4>	not report periodic CSI that is not L1-RSRP on PUCCH.

1>	else:

2>	in current symbol n, if the DRX group would not be in Active Time considering grants/assignments scheduled on Serving Cell(s) in this DRX Group and DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause:

3>	not transmit periodic SRS and semi-persistent SRS defined in TS 38.214 [7] in this DRX group;

3>	not report CSI on PUCCH and semi-persistent CSI configured on PUSCH in this DRX group.

…

Regardless of whether the MAC entity is monitoring PDCCH or not on the Serving Cells in this DRX group, the MAC entity transmits HARQ feedback, aperiodic CSI on PUSCH, and aperiodic SRS defined in TS 38.214 [7] on the Serving Cells in this DRX group when such is expected.

The MAC entity needs not to monitor the PDCCH if it is not a complete PDCCH occasion (e.g. the Active Time starts or ends in the middle of a PDCCH occasion).

[TS 38.213, clause 10.3]

A UE configured with DRX mode operation [11, TS 38.321] can be provided the following for detection of a DCI format 2_6 in a PDCCH reception on the PCell or on the SpCell [12, TS 38.331]

-	a PS-RNTI for DCI format 2_6 by ps-RNTI

-	a number of search space sets, by dci-Format2-6, to monitor PDCCH for detection of DCI format 2_6 on the active DL BWP of the PCell or of the SpCell according to a common search space as described in clause 10.1

-	a payload size for DCI format 2_6 by sizeDCI-2-6

-	a location in DCI format 2_6 of a Wake-up indication bit by ps-PositionDCI-2-6

-	a '0' value for the Wake-up indication bit, when reported to higher layers, indicates to not start the drx-onDurationTimer for the next long DRX cycle [11, TS 38.321]

-	a '1' value for the Wake-up indication bit, when reported to higher layers, indicates to start the drx-onDurationTimer for the next long DRX cycle [11, TS 38.321]

-	a bitmap, when the UE is provided a number of groups of configured SCells by dormancyGroupOutsideActiveTime, where

-	the bitmap location is immediately after the Wake-up indication bit location

-	the bitmap size is equal to the number of groups of configured SCells where each bit of the bitmap corresponds to a group of configured SCells from the number of groups of configured SCells

-	a '0' value for a bit of the bitmap indicates an active DL BWP, provided by dormantBWP-Id, for the UE [11, TS38.321] for each activated SCell in the corresponding group of configured SCells

-	a '1' value for a bit of the bitmap indicates

-	an active DL BWP, provided by firstOutsideActiveTimeBWP-Id, for the UE for each activated SCell in the corresponding group of configured SCells, if a current active DL BWP is the dormant DL BWP

-	a current active DL BWP, for the UE for each activated SCell in the corresponding group of configured SCells, if the current active DL BWP is not the dormant DL BWP

-	the UE sets the active DL BWP to the indicated active DL BWP

-	an offset by ps-Offset indicating a time, where the UE starts monitoring PDCCH for detection of DCI format 2_6 according to the number of search space sets, prior to a slot where the drx-onDurationTimer would start on the PCell or on the SpCell [11, TS 38.321]

-	for each search space set, the PDCCH monitoring occasions are the ones in the first  slots indicated by duration, or  slot if duration is not provided, starting from the first slot of the first  slots and ending prior to the start of drx-onDurationTimer.

On PDCCH monitoring occasions associated with a same long DRX Cycle, a UE does not expect to detect more than one DCI format 2_6 with different values of the Wake-up indication bit for the UE or with different values of the bitmap for the UE.

The UE does not monitor PDCCH for detecting DCI format 2_6 during Active Time [11, TS 38.321].

If a UE reports for an active DL BWP a MinTimeGap value that is X slots prior to the beginning of a slot where the UE would start the drx-onDurationTimer, the UE is not required to monitor PDCCH for detection of DCI format 2_6 during the X slots, where X corresponds to the MinTimeGap value of the SCS of the active DL BWP in Table 10.3-1.

Table 10.3-1: Minimum time gap value X

| SCS (kHz) | Minimum Time Gap X (slots) |  |
| --- | --- | --- |
|  | Value 1 | Value 2 |
| 15 | 1 | 3 |
| 30 | 1 | 6 |
| 60 | 1 | 12 |
| 120 | 2 | 24 |
| 480 | 8 | 96 |
| 960 | 16 | 192 |

If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE detects DCI format 2_6, the physical layer of a UE reports the value of the Wake-up indication bit for the UE to higher layers [11, TS 38.321] for the next long DRX cycle.

If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE does not detect DCI format 2_6, the physical layer of the UE does not report a value of the Wake-up indication bit to higher layers for the next long DRX cycle.

If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE

-	is not required to monitor PDCCH for detection of DCI format 2_6, as described in clauses 10, 11.1, 12, and in clause 5.7 of [11, TS 38.321] for all corresponding PDCCH monitoring occasions outside Active Time prior to a next long DRX cycle, or

-	does not have any PDCCH monitoring occasions for detection of DCI format 2_6 outside Active Time of a next long DRX cycle

the physical layer of the UE reports a value of 1 for the Wake-up indication bit to higher layers for the next long DRX cycle.

[TS 38.212, clause 7.3.1.3.7]

DCI format 2_6 is used for notifying the power saving information outside DRX Active Time for one or more UEs.

The following information is transmitted by means of the DCI format 2_6 with CRC scrambled by PS-RNTI:

-	block number 1, block number 2,…, block number N

where the starting position of a block is determined by the parameter PSPositionDCI2-6 provided by higher layers for the UE configured with the block.

If the UE is configured with higher layer parameter PS-RNTI and dci-Format2-6, one block is configured for the UE by higher layers, with the following fields defined for the block:

-	Wake-up indication - 1 bit

-	SCell dormancy indication – 0 bit if higher layer parameter Scell-groups-for-dormancy-outside-active-time is not configured; otherwise 1, 2, 3, 4 or 5 bits bitmap determined according to higher layer parameter Scell-groups-for-dormancy-outside-active-time, where each bit corresponds to one of the SCell group(s) configured by higher layers parameter Scell-groups-for-dormancy-outside-active-time, with MSB to LSB of the bitmap corresponding to the first to last configured SCell group.

The size of DCI format 2_6 is indicated by the higher layer parameter SizeDCI_2-6, according to Clause 10.3 of [5, TS 38.213].

7.1.1.12.3.3	Test description

7.1.1.12.3.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that set to return no data in uplink.

7.1.1.12.3.3.2	Test procedure sequence

Table 7.1.1.12.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits RRCReconfiguration to configure specific DCP parameters. (Note 1) | <-- | RRCReconfiguration | - | - |
| 2 | The UE transmits RRCReconfigurationComplete. (Note 2) | --> | RRCReconfigurationComplete | - | - |
| 3 | Wait 1280ms to ensure UE is out DRX active time. | - | - | - | - |
| 3A | The SS transmits DCI 2-6 on the PDCCH within the PS-offset time before the start of next long DRX drx-onDurationTimer and the DCI 2-6 indicates not to start the next Drx-onDurationTimer. | <-- | (PDCCH (DCI 2-6)) | - | - |
| 3B | In a PDCCH occasion the SS indicates the transmission of a DL MAC PDU on the PDCCH. | <-- | MAC PDU | - | - |
| 3C | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 3B? | --> | HARQ ACK | 5 | F |
| 4 | The SS transmits DCI 2-6 on the PDCCH within the PS-offset time before the start of next long DRX drx-onDurationTimer and the DCI 2-6 indicates to start the next Drx-onDurationTimer. | <-- | (PDCCH (DCI 2-6)) | - | - |
| 5 | In the first PDCCH occasion when the Drx-onDurationTimer is running, the SS indicates the transmission of a DL MAC PDU on the PDCCH. | <-- | MAC PDU | - | - |
| 6 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 5? | --> | HARQ ACK | 1 | P |
| 7 | The SS transmits RRCReconfiguration to configure ps-wakeup with value true. (Note 1) | <-- | RRCReconfiguration | - | - |
| 8 | The UE transmits RRCReconfigurationComplete. (Note 2) | --> | RRCReconfigurationComplete | - | - |
| 9 | Wait 1280ms to ensure UE is out DRX active time. | - | - | - | - |
| 10 | In the first PDCCH occasion when the Drx-onDurationTimer is running, the SS indicates the transmission of a DL MAC PDU on the PDCCH. | <-- | MAC PDU | - | - |
| 11 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 10? | --> | HARQ ACK | 2 | P |
| 12 | SS transmits RRCReconfiguration to configure specific DCP parameters. (Note 1) | <-- | RRCReconfiguration | - | - |
| 13 | The UE transmits RRCReconfigurationComplete. (Note 2) | --> | RRCReconfigurationComplete | - | - |
| 14 | Wait 400ms to ensure UE is out DRX active time. | - | - | - | - |
| 15 | The SS transmits DCI 2-6 on the PDCCH within the PS-offset time before the start of next long DRX drx-onDurationTimer and the DCI 2-6 indicates to start the next Drx-onDurationTimer. | <-- | (PDCCH (DCI 2-6)) | - | - |
| 16 | In the last PDCCH occasion when the Drx-onDurationTimer is running, the SS indicates the transmission of an invalid DL MAC PDU on the PDCCH. | <-- | Invalid MAC PDU | - | - |
| 17 | The UE transmits a HARQ NACK for the DL MAC PDU in Step 16. | --> | HARQ NACK | - | - |
| 17A | The SS transmits DCI 2-6 on the PDCCH within the PS-offset time before the start of next long DRX drx-onDurationTimer and the DCI 2-6 indicates not to start the next Drx-onDurationTimer. | <-- | (PDCCH (DCI 2-6)) | - | - |
| 18 | In the PDCCH occasion when the next Drx-onDurationTimer is running, the SS indicates the transmission of a DL MAC PDU on the PDCCH. | <-- | MAC PDU | - | - |
| 19 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 18? | --> | HARQ ACK | 3 | P |
| 19A | The SS transmits DCI 2-6 on the PDCCH within the PS-offset time before the start of next long DRX drx-onDurationTimer and the DCI 2-6 indicates to start the next Drx-onDurationTimer. | <-- | (PDCCH (DCI 2-6)) | - | - |
| 20 | The SS transmits RRCReconfiguration to configure specific measonfig parameters. (Note 1) | <-- | RRCReconfiguration | - | - |
| 21 | The UE transmits RRCReconfigurationComplete. (Note 2) | --> | RRCReconfigurationComplete | - | - |
| 22 | Wait 10ms to ensure UE is out DRX active time. | - | - | - | - |
| 22A | The SS transmits DCI 2-6 on the PDCCH within the PS-offset time before the start of next long DRX drx-onDurationTimer and the DCI 2-6 indicates not to start the next Drx-onDurationTimer. | <-- | (PDCCH (DCI 2-6)) | - | - |
| 23 | In the first PDCCH occasion when the Drx-onDurationTimer is running, the SS indicates the transmission of a DL MAC PDU on the PDCCH. | <-- | MAC PDU | - | - |
| 24 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 23? | --> | HARQ ACK | 4 | P |
| Note 1: For EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration 36.508 [7], Table 4.6.1-8 using condition EN-DC_EmbedNR_RRCRecon.In addition to this, for Step 20, the specific message contents in Table 7.1.1.12.3.3.3-9 for RRC Connection Reconfiguration is used Note 2: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. |  |  |  |  |  |

7.1.1.12.3.3.3	Specific message contents

Table 7.1.1.12.3.3.3-1: RRCReconfiguration (steps 1, 7 and 12, Table 7.1.1.12.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| secondaryCellGroup |  | CellGroupConfig |  | EN-DC |
| nonCriticalExtension |  | Not present |  | EN-DC |
| nonCriticalExtension SEQUENCE { |  |  |  | NR |
| masterCellGroup |  | CellGroupConfig |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.12.3.3.3-2: CellGroupConfig (Table 7.1.1.12.3.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated | ServingCellConfig |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.12.3.3.3-3: ServingCellConfig (Table 7.1.1.13.3.3.3-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP ::= SEQUENCE { |  |  |  |
| pdcch-Config CHOICE { |  |  | Step 1, Step 7, Step 12, Step 20 |
| setup | PDCCH-Config |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.12.3.3.3-4: PDCCH-Config (Table 7.1.1.12.3.3.3-3)

Derivation Path: TS 38.508-1 [4],Table 4.6.3-95 with condition DCI_2_6

Table 7.1.1.12.3.3.3-5: CellGroupConfig (Table 7.1.1.13.3.3.3-1: RRCReconfiguration step 7)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| physicalCellGroupConfig::= SEQUENCE { |  |  |  |
| dcp-Config-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| ps-WakeUp-r16 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.12.3.3.3-6: CellGroupConfig (Table 7.1.1.13.3.3.3-1: RRCReconfiguration step 12)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| mac-CellGroupConfig ::= SEQUENCE { |  |  |  |
| drx-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| drx-onDurationTimer CHOICE { | ms10 |  |  |
| milliSeconds | ms10 |  |  |
| } |  |  |  |
| drx-InactivityTimer | ms6 |  |  |
| drx-HARQ-RTT-TimerDL | 56 |  |  |
| drx-HARQ-RTT-TimerUL | 56 |  |  |
| drx-RetransmissionTimerDL | sl320 |  |  |
| drx-RetransmissionTimerUL | sl320 |  |  |
| drx-LongCycleStartOffset CHOICE { |  |  |  |
| ms20 | 0 |  |  |
| } |  |  |  |
| shortDRX | Not present |  |  |
| drx-SlotOffset | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| physicalCellGroupConfig::= SEQUENCE { |  |  |  |
| dcp-Config-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| ps-Offset-r16 | 40 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.12.3.3.3-7: RRCReconfiguration (step 20, Table 7.1.1.12.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| secondaryCellGroup |  | CellGroupConfig |  | EN-DC |
| measConfig ::= SEQUENCE { |  |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { |  | 2 entries |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  |  | entry 1 |  |
| measObjectId |  | 1 |  |  |
| measObject CHOICE { |  |  |  |  |
| measObjectNR SEQUENCE { |  |  |  |  |
| ssbFrequency |  | ARFCN-ValueNR for SSB of NR Cell 1 |  |  |
| absThreshSS-BlocksConsolidation |  | Not present |  |  |
| nrofSS-BlocksToAverage |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| MeasObjectToAddMod[2] SEQUENCE { |  |  |  |  |
| measObjectId |  | 2 |  |  |
| measObject CHOICE { |  |  |  |  |
| measObjectNR SEQUENCE { |  |  |  |  |
| ssbFrequency |  | ARFCN-ValueNR for SSB of NR Cell 3 |  |  |
| absThreshSS-BlocksConsolidation |  | Not present |  |  |
| nrofSS-BlocksToAverage |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { |  | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  |  | entry 1 |  |
| reportConfigId |  | 1 |  |  |
| reportConfig CHOICE { |  |  |  |  |
| reportConfigNR SEQUENCE { |  |  |  |  |
| reportType CHOICE { |  |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |  |
| eventId CHOICE { |  |  |  |  |
| eventA3 SEQUENCE { |  |  |  |  |
| a3-Offset CHOICE { |  |  |  |  |
| rsrp |  | 2 | 1 dB (2*0.5 dB) |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| reportAmount |  | infinity |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |  |
| rsrp |  | true |  |  |
| rsrq |  | false |  |  |
| sinr |  | false |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { |  | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  |  | entry 1 |  |
| measId |  | 1 |  |  |
| measObjectId |  | 2 |  |  |
| reportConfigId |  | 1 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| measGapConfig ::= SEQUENCE { |  |  |  |  |
| gapUE CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| gapOffset |  | 34 |  |  |
| mgl |  | ms6 |  |  |
| mgrp |  | ms40 |  |  |
| mgta |  | ms0 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | NR |
| masterCellGroup |  | CellGroupConfig |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.12.3.3.3-8: CellGroupConfig (Table 7.1.1.12.3.3.3-7)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| mac-CellGroupConfig ::= SEQUENCE { |  |  |  |
| drx-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| drx-onDurationTimer CHOICE { |  |  |  |
| milliSeconds | ms10 |  |  |
| } |  |  |  |
| drx-InactivityTimer | ms5 |  |  |
| drx-LongCycleStartOffset CHOICE { |  |  |  |
| ms40 | 39 |  |  |
| } |  |  |  |
| shortDRX | Not present |  |  |
| drx-SlotOffset | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| physicalCellGroupConfig::= SEQUENCE { |  |  |  |
| dcp-Config-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| ps-Offset-r16 | 32 |  |  |
| ps-PositionDCI-2-6-r16 | 5 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.12.3.3.3-9: RRCConnectionReconfiguration (Step 20)

| Derivation path: 36.508 table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| measConfig SEQUENCE { |  |  |  |
| measGapConfig CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| gapOffset CHOICE { |  |  |  |
| gp0 | 34 | MGRP = 40 ms |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 7.1.1.12.4	DRX adaptation / SCell dormancy indication

###### 7.1.1.12.4.1	DRX adaptation / SCell dormancy indication / Intra-band Contiguous CA

7.1.1.12.4.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with SCell configured and long DRX is configured and DCP is configured }

ensure that {

when { UE is outside DRX active time and receives the PDCCH indicating entering dormant BWP for SCell }

then { UE activates the BWP indicated by dormantBWP-Id and stops monitoring the PDCCH }

}

(2)

with { UE in RRC_CONNECTED state with SCell configured and long DRX is configured and DCP is configured }

ensure that {

when { UE is outside DRX active time and the active DL BWP is dormant BWP and receives the PDCCH indicating leaving dormant BWP from SCell }

then { UE activates the BWP indicated by firstOutsideActiveTimeBWP-Id and starts normal MAC operation on the new BWP }

}

7.1.1.12.4.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.212, clause 7.3.1.3.7, TS 38.213, clause 10.3, TS 38.321, clause 5.15.1 and 5.9. Unless otherwise stated these are Rel-16 requirements.

[TS 38.212, clause 7.3.1.3.7]

DCI format 2_6 is used for notifying the power saving information outside DRX Active Time for one or more UEs.

The following information is transmitted by means of the DCI format 2_6 with CRC scrambled by PS-RNTI:

-	block number 1, block number 2,…, block number N

where the starting position of a block is determined by the parameter ps-PositionDCI-2-6 provided by higher layers for the UE configured with the block.

If the UE is configured with higher layer parameter ps-RNTI and dci-Format2-6, one block is configured for the UE by higher layers, with the following fields defined for the block:

-	Wake-up indication - 1 bit

-	SCell dormancy indication – 0 bit if higher layer parameter dormancyGroupOutsideActiveTime is not configured; otherwise 1, 2, 3, 4 or 5 bits bitmap determined according to higher layer parameter dormancyGroupOutsideActiveTime, where each bit corresponds to one of the SCell group(s) configured by higher layers parameter dormancyGroupOutsideActiveTime, with MSB to LSB of the bitmap corresponding to the first to last configured SCell group.

The size of DCI format 2_6 is indicated by the higher layer parameter sizeDCI-2-6, according to Clause 10.3 of [5, TS 38.213].

[TS 38.213, clause 10.3]

A UE configured with DRX mode operation [11, TS 38.321] can be provided the following for detection of a DCI format 2_6 in a PDCCH reception on the PCell or on the SpCell [12, TS 38.331]

-	a PS-RNTI for DCI format 2_6 by ps-RNTI

-	a number of search space sets, by dci-Format2-6, to monitor PDCCH for detection of DCI format 2_6 on the active DL BWP of the PCell or of the SpCell according to a common search space as described in Clause 10.1

-	a payload size for DCI format 2_6 by sizeDCI_2-6

-	a location in DCI format 2_6 of a Wake-up indication bit by psPositionDCI-2-6

-	a '0' value for the Wake-up indication bit, when reported to higher layers, indicates to not start the drx-onDurationTimer for the next long DRX cycle [11, TS 38.321]

-	a '1' value for the Wake-up indication bit, when reported to higher layers, indicates to start the drx-onDurationTimer for the next long DRX cycle [11, TS 38.321]

-	a bitmap, when the UE is provided a number of groups of configured SCells by dormancyGroupOutsideActiveTime, where

-	the bitmap location is immediately after the Wake-up indication bit location

-	the bitmap size is equal to the number of groups of configured SCells where each bit of the bitmap corresponds to a group of configured SCells from the number of groups of configured SCells

-	a '0' value for a bit of the bitmap indicates an active DL BWP, provided by dormantBWP-Id, for the UE [11, TS38.321] for each activated SCell in the corresponding group of configured SCells

-	a '1' value for a bit of the bitmap indicates

-	an active DL BWP, provided by firstOutsideActiveTimeBWP-Id, for the UE for each activated SCell in the corresponding group of configured SCells, if a current active DL BWP is the dormant DL BWP

-	a current active DL BWP, for the UE for each activated SCell in the corresponding group of configured SCells, if the current active DL BWP is not the dormant DL BWP

-	an offset by ps-Offset indicating a time, where the UE starts monitoring PDCCH for detection of DCI format 2_6 according to the number of search space sets, prior to a slot where the drx-onDuarationTimer would start on the PCell or on the SpCell [11, TS 38.321]

-	for each search space set, the PDCCH monitoring occasions are the ones in the first  slots indicated by duration, or  slot if duration is not provided, starting from the first slot of the first  slots and ending prior to the start of drx-onDurationTimer.

On PDCCH monitoring occasions associated with a same long DRX Cycle, a UE does not expect to detect more than one DCI format 2_6 with different values of the Wake-up indication bit for the UE or with different values of the bitmap for the UE.

The UE does not monitor PDCCH for detecting DCI format 2_6 during Active Time [11, TS 38.321].

If a UE reports for an active DL BWP a requirement of X slots prior to the beginning of a slot where the UE would start the drx-onDurationTimer, the UE is not required to monitor PDCCH for detection of DCI format 2_6 during the X slots, where X corresponds to the requirement of the SCS of the active DL BWP in Table 10.3-1.

Table 10.3-1: Minimum time gap value X

| SCS (kHz) | Minimum Time Gap X (slots) |  |
| --- | --- | --- |
|  | Value 1 | Value 2 |
| 15 | 1 | 3 |
| 30 | 1 | 6 |
| 60 | 1 | 12 |
| 120 | 2 | 24 |

If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE detects DCI format 2_6, the physical layer of a UE reports the value of the Wake-up indication bit for the UE to higher layers [11, TS 38.321] for the next long DRX cycle.

If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE does not detect DCI format 2_6, the physical layer of the UE does not report a value of the Wake-up indication bit to higher layers for the next long DRX cycle.

If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE

-	is not required to monitor PDCCH for detection of DCI format 2_6, as described in Clauses 10, 11.1, 12, and in Clause 5.7 of [11, TS 38.321] for all corresponding PDCCH monitoring occasions outside Active Time prior to a next long DRX cycle, or

-	does not have any PDCCH monitoring occasions for detection of DCI format 2_6 outside Active Time of a next long DRX cycle

the physical layer of the UE reports a value of 1 for the Wake-up indication bit to higher layers for the next long DRX cycle.

…

If an active DL BWP provided by dormantBWP-Id for a UE on an activated SCell is not a default DL BWP for the UE on the activated SCell, as described in Clause 12, the BWP inactivity timer is not used for transitioning from the active DL BWP provided by dormantBWP-Id to the default DL BWP on the activated SCell.

[TS 38.321, clause 5.15.1]

In addition to clause 12 of TS 38.213 [6], this clause specifies requirements on BWP operation.

A Serving Cell may be configured with one or multiple BWPs, and the maximum number of BWP per Serving Cell is specified in TS 38.213 [6].

The BWP switching for a Serving Cell is used to activate an inactive BWP and deactivate an active BWP at a time. The BWP switching is controlled by the PDCCH indicating a downlink assignment or an uplink grant, by the bwp-InactivityTimer, by RRC signalling, or by the MAC entity itself upon initiation of Random Access procedure or upon detection of consistent LBT failure on SpCell. Upon RRC (re-)configuration of firstActiveDownlinkBWP-Id and/or firstActiveUplinkBWP-Id for SpCell or activation of an SCell, the DL BWP and/or UL BWP indicated by firstActiveDownlinkBWP-Id and/or firstActiveUplinkBWP-Id respectively (as specified in TS 38.331 [5]) is active without receiving PDCCH indicating a downlink assignment or an uplink grant. The active BWP for a Serving Cell is indicated by either RRC or PDCCH (as specified in TS 38.213 [6]). For unpaired spectrum, a DL BWP is paired with a UL BWP, and BWP switching is common for both UL and DL.

For each SCell a dormant BWP may be configured with dormantBWP-Id by RRC signalling as described in TS 38.331 [5]. Entering or leaving dormant BWP for SCells is done by BWP switching per SCell or per dormancy SCell group based on instruction from PDCCH (as specified in TS 38.213 [6]). The dormancy SCell group configurations are configured by RRC signalling as described in TS 38.331 [5]. Upon reception of the PDCCH indicating leaving dormant BWP, the DL BWP indicated by firstOutsideActiveTimeBWP-Id or by firstWithinActiveTimeBWP-Id (as specified in TS 38.331 [5] and TS 38.213 [6]) is activated. Upon reception of the PDCCH indicating entering dormant BWP, the DL BWP indicated by dormantBWP-Id (as specified in TS 38.331 [5]) is activated. The dormant BWP configuration for SpCell or PUCCH SCell is not supported.

For each activated Serving Cell configured with a BWP, the MAC entity shall:

1>	if a BWP is activated and the active DL BWP for the Serving Cell is not the dormant BWP:

2>	transmit on UL-SCH on the BWP;

2>	transmit on RACH on the BWP, if PRACH occasions are configured;

2>	monitor the PDCCH on the BWP;

2>	transmit PUCCH on the BWP, if configured;

2>	report CSI for the BWP;

2>	transmit SRS on the BWP, if configured;

2>	receive DL-SCH on the BWP;

2>	(re-)initialize any suspended configured uplink grants of configured grant Type 1 on the active BWP according to the stored configuration, if any, and to start in the symbol according to rules in clause 5.8.2;

2>	if lbt-FailureRecoveryConfig is configured:

3>	stop the lbt-FailureDetectionTimer, if running;

3>	set LBT_COUNTER to 0;

3>	monitor LBT failure indications from lower layers as specified in clause 5.21.2.

1>	if a BWP is activated and the active DL BWP for the Serving Cell is dormant BWP:

2>	stop the bwp-InactivityTimer of this Serving Cell, if running.

2>	not monitor the PDCCH on the BWP;

2>	not monitor the PDCCH for the BWP;

2>	not receive DL-SCH on the BWP;

2>	not report CSI on the BWP, report CSI except aperiodic CSI for the BWP;

2>	not transmit SRS on the BWP;

2>	not transmit on UL-SCH on the BWP;

2>	not transmit on RACH on the BWP;

2>	not transmit PUCCH on the BWP.

2>	clear any configured downlink assignment and any configured uplink grant Type 2 associated with the SCell respectively;

2>	suspend any configured uplink grant Type 1 associated with the SCell;

2>	if configured, perform beam failure detection and beam failure recovery for the SCell if beam failure is detected.

1>	if a BWP is deactivated:

2>	not transmit on UL-SCH on the BWP;

2>	not transmit on RACH on the BWP;

2>	not monitor the PDCCH on the BWP;

2>	not transmit PUCCH on the BWP;

2>	not report CSI for the BWP;

2>	not transmit SRS on the BWP;

2>	not receive DL-SCH on the BWP;

2>	clear any configured downlink assignment and configured uplink grant of configured grant Type 2 on the BWP;

2>	suspend any configured uplink grant of configured grant Type 1 on the inactive BWP.

Upon initiation of the Random Access procedure on a Serving Cell, after the selection of carrier for performing Random Access procedure as specified in clause 5.1.1, the MAC entity shall for the selected carrier of this Serving Cell:

1>	if PRACH occasions are not configured for the active UL BWP:

2>	switch the active UL BWP to BWP indicated by initialUplinkBWP;

2>	if the Serving Cell is an SpCell:

3>	switch the active DL BWP to BWP indicated by initialDownlinkBWP.

1>	else:

2>	if the Serving Cell is an SpCell:

3>	if the active DL BWP does not have the same bwp-Id as the active UL BWP:

4>	switch the active DL BWP to the DL BWP with the same bwp-Id as the active UL BWP.

1>	stop the bwp-InactivityTimer associated with the active DL BWP of this Serving Cell, if running.

1>	if the Serving Cell is SCell:

2>	stop the bwp-InactivityTimer associated with the active DL BWP of SpCell, if running.

1>	perform the Random Access procedure on the active DL BWP of SpCell and active UL BWP of this Serving Cell.

If the MAC entity receives a PDCCH for BWP switching of a Serving Cell, the MAC entity shall:

1>	if there is no ongoing Random Access procedure associated with this Serving Cell; or

1>	if the ongoing Random Access procedure associated with this Serving Cell is successfully completed upon reception of this PDCCH addressed to C-RNTI (as specified in clauses 5.1.4, 5.1.4a, and 5.1.5):

2>	cancel, if any, triggered consistent LBT failure for this Serving Cell;

2>	perform BWP switching to a BWP indicated by the PDCCH.

If the MAC entity receives a PDCCH for BWP switching for a Serving Cell(s) or a dormancy SCell group(s) while a Random Access procedure associated with that Serving Cell is ongoing in the MAC entity, it is up to UE implementation whether to switch BWP or ignore the PDCCH for BWP switching, except for the PDCCH reception for BWP switching addressed to the C-RNTI for successful Random Access procedure completion (as specified in clauses 5.1.4, 5.1.4a, and 5.1.5) in which case the UE shall perform BWP switching to a BWP indicated by the PDCCH. Upon reception of the PDCCH for BWP switching other than successful contention resolution, if the MAC entity decides to perform BWP switching, the MAC entity shall stop the ongoing Random Access procedure and initiate a Random Access procedure after performing the BWP switching; if the MAC decides to ignore the PDCCH for BWP switching, the MAC entity shall continue with the ongoing Random Access procedure on the Serving Cell.

…

1>	if a PDCCH for BWP switching is received, and the MAC entity switches the active DL BWP:

2>	if the defaultDownlinkBWP-Id is configured, and the MAC entity switches to the DL BWP which is not indicated by the defaultDownlinkBWP-Id and is not indicated by the dormantBWP-Id if configured; or

2>	if the defaultDownlinkBWP-Id is not configured, and the MAC entity switches to the DL BWP which is not the initialDownlinkBWP and is not indicated by the dormantBWP-Id if configured:

3>	start or restart the bwp-InactivityTimer associated with the active DL BWP.

[TS 38.321, clause 5.9]

If the MAC entity is configured with one or more SCells, the network may activate and deactivate the configured SCells. Upon configuration of an SCell, the SCell is deactivated unless the parameter sCellState is set to activated for the SCell by upper layers.

The configured SCell(s) is activated and deactivated by:

-	receiving the SCell Activation/Deactivation MAC CE described in clause 6.1.3.10;

-	configuring sCellDeactivationTimer timer per configured SCell (except the SCell configured with PUCCH, if any): the associated SCell is deactivated upon its expiry;

-	configuring sCellState per configured SCell: if configured, the associated SCell is activated upon SCell configuration.

The MAC entity shall for each configured SCell:

1>	if an SCell is configured with sCellState set to activated upon SCell configuration, or an SCell Activation/Deactivation MAC CE is received activating the SCell:

2>	if the SCell was deactivated prior to receiving this SCell Activation/Deactivation MAC CE; or

2>	if the SCell is configured with sCellState set to activated upon SCell configuration:

3>	if firstActiveDownlinkBWP-Id is not set to dormant BWP:

4>	activate the SCell according to the timing defined in TS 38.213 [6]; i.e. apply normal SCell operation including:

5>	SRS transmissions on the SCell;

5>	CSI reporting for the SCell;

5>	PDCCH monitoring on the SCell;

5>	PDCCH monitoring for the SCell;

5>	PUCCH transmissions on the SCell, if configured.

3>	else (i.e. firstActiveDownlinkBWP-Id is set to dormant BWP):

4>	stop the bwp-InactivityTimer of this Serving Cell, if running.

3>	activate the DL BWP and UL BWP indicated by firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id respectively.

2>	start or restart the sCellDeactivationTimer associated with the SCell according to the timing defined in TS 38.213 [6];

2>	if the active DL BWP is not the dormant BWP:

3>	(re-)initialize any suspended configured uplink grants of configured grant Type 1 associated with this SCell according to the stored configuration, if any, and to start in the symbol according to rules in clause 5.8.2.2;

3>	trigger PHR according to clause 5.4.6.

1>	else if an SCell Activation/Deactivation MAC CE is received deactivating the SCell; or

1>	if the sCellDeactivationTimer associated with the activated SCell expires:

2>	deactivate the SCell according to the timing defined in TS 38.213 [6];

2>	stop the sCellDeactivationTimer associated with the SCell;

2>	stop the bwp-InactivityTimer associated with the SCell;

2>	deactivate any active BWP associated with the SCell;

2>	clear any configured downlink assignment and any configured uplink grant Type 2 associated with the SCell respectively;

2>	clear any PUSCH resource for semi-persistent CSI reporting associated with the SCell;

2>	suspend any configured uplink grant Type 1 associated with the SCell;

2>	flush all HARQ buffers associated with the SCell;

2>	cancel, if any, triggered consistent LBT failure for the SCell.

1>	if PDCCH on the activated SCell indicates an uplink grant or downlink assignment; or

1>	if PDCCH on the Serving Cell scheduling the activated SCell indicates an uplink grant or a downlink assignment for the activated SCell; or

1>	if a MAC PDU is transmitted in a configured uplink grant and LBT failure indication is not received from lower layers; or

1>	if a MAC PDU is received in a configured downlink assignment:

2>	restart the sCellDeactivationTimer associated with the SCell.

1>	if the SCell is deactivated:

2>	not transmit SRS on the SCell;

2>	not report CSI for the SCell;

2>	not transmit on UL-SCH on the SCell;

2>	not transmit on RACH on the SCell;

2>	not monitor the PDCCH on the SCell;

2>	not monitor the PDCCH for the SCell;

2>	not transmit PUCCH on the SCell.

HARQ feedback for the MAC PDU containing SCell Activation/Deactivation MAC CE shall not be impacted by PCell, PSCell and PUCCH SCell interruptions due to SCell activation/deactivation in TS 38.133 [11].

When SCell is deactivated, the ongoing Random Access procedure on the SCell, if any, is aborted.

7.1.1.12.4.1.3	Test description

7.1.1.12.4.1.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that set to return no data in uplink. System information combination NR-4 and in addition NR Cell 3 is configured as NR Active SCell.

7.1.1.12.4.1.3.2	Test procedure sequence

Table 7.1.1.12.4.1.3.2-1: Cell configuration power level changes over time for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 | Remarks |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | off | NR cell 1 is available and NR cell 3 is not available |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -88 | NR cell 1 and NR cell 3 are available |

Table 7.1.1.12.4.1.3.2-2: Cell configuration power level changes over time for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 | Remarks |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -82 | off | NR cell 1 is available and NR cell 3 is not available |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -82 | NR cell 1 and NR cell 3 are available |

Table 7.1.1.12.4.1.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | Set the power levels according to “T1” as per Table 7.1.1.12.4.1.3.2-1/2. |  | - | - | - |
| 1 | SS transmits an RRCReconfiguration message. (Note 1) | <-- | - | - | - |
| 2 | The UE transmits RRCReconfigurationComplete message. (Note 2) | --> | - | - | - |
| 3 | The SS transmits a SCell Activation MAC-CE to activate SCell (NR Cell 3). | <-- | MAC PDU (SCell Activation/Deactivation MAC CE of one octet (C1=1)) | - | - |
| 4 | The SS transmits DCI 2-6 within ps-Offset time before the start of next long DRX drx-onDurationTimer on NR Cell 1. (Note 3) | <-- | (PDCCH (DCI 2-6)) | - | - |
| 5 | The SS indicates a new transmission on PDCCH of SCell and transmits a MAC PDU on the initial BWP (BWP#0) when the Drx-onDurationTimer is running. | <-- | MAC PDU | - | -- |
| 6 | Check: Does the UE transmit a HARQ ACK on the PCell for the DL MAC PDU in Step 5 within 5 seconds? | --> | - | 1 | F |
| 7 | The SS transmits DCI 2-6 within the ps-offset time before the start of next long DRX drx-onDurationTimer on NR Cell 1. (Note 4) | <-- | (PDCCH (DCI 2-6)) | - | - |
| 8 | The SS indicates a new transmission on PDCCH of SCell and transmits a MAC PDU on the active BWP (BWP#0) when the Drx-onDurationTimer is running. | <-- | MAC PDU | - | - |
| 9 | Check: Does the UE transmit a HARQ ACK on the PCell NR Cell 1 for the DL MAC PDU in Step 8? | --> | HARQ ACK | 2 | P |
| Note 1: For EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration TS 36.508 [7], Table 4.6.1-8 using condition EN-DC_EmbedNR_RRCRecon. Note 2: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. Note 3: The Wake-up indication is value 1 and the SCell dormancy indication is value 0 in the DCI 2-6. Note 4: The Wake-up indication is value 1 and the SCell dormancy indication is value 1 in the DCI 2-6. |  |  |  |  |  |

7.1.1.12.4.1.3.3	Specific message contents

Table 7.1.1.12.4.1.3.3-1: RRCReconfiguration (step 1, Table 7.1.1.12.4.1.3.2-3)

| Derivation Path: TS 38.508-1 [6], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| secondaryCellGroup |  | CellGroupConfig |  | EN-DC |
|  |  | Not present |  | NR |
| nonCriticalExtension |  | Not present |  | EN-DC |
| nonCriticalExtension SEQUENCE { |  |  |  | NR |
| masterCellGroup |  | CellGroupConfig |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.12.4.1.3.3-2: CellGroupConfig (Table 7.1.1.12.4.1.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |  |
| cellGroupId |  | CellGroupId | TS 38.508-1 default value |  |
| mac-CellGroupConfig SEQUENCE { |  |  |  |  |
| drx-Config CHOICE { |  |  |  |  |
| setup |  | DRX-Config | TS 38.508-1 default value |  |
| } |  |  |  |  |
| } |  |  |  |  |
| physicalCellGroupConfig SEQUENCE { |  |  |  |  |
| dcp-Config-r16 CHOICE { |  |  |  |  |
| setup |  | DCP-Config-r16 | TS 38.508-1 default value |  |
| } |  |  |  |  |
| } |  |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |  |
| initialDownlinkBWP SEQUENCE { |  |  |  |  |
| pdcch-Config CHOICE { |  |  |  |  |
| setup |  | PDCCH-Config |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SCellConfig { |  | 1 entry |  |  |
| SCellConfig[1] SEQUENCE { |  |  | entry 1 |  |
| sCellIndex |  | SCellIndex as per TS 38.508-1 [4] table 4.6.3-154 |  |  |
| sCellConfigCommon |  | ServingCellConfigCommon |  |  |
| sCellConfigDedicated |  | ServingCellConfig |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.12.4.1.3.3-3: PDCCH-Config (Table 7.1.1.12.4.1.3.3-2)

Derivation Path: TS 38.508-1 [4],Table 4.6.3-95 with condition DCI_2_6

Table 7.1.1.12.4.1.3.3-4: ServingCellConfigCommon (Table 7.1.1.12.4.1.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-168 with conditions No_UL and SCell_add. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| physCellId | Physical Cell Identity of NR Cell 3 |  |  |
| } |  |  |  |

Table 7.1.1.12.4.1.3.3-5: ServingCellConfig (Table 7.1.1.12.4.1.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-167 with conditions No_UL and SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) BWP-Downlink { |  |  |  |
| BWP-Downlink | BWP-Downlink |  |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 0 |  |  |
| dormantBWP-Config-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| dormantBWP-Id-r16 | 1 |  |  |
| outsideActiveTimeConfig-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| firstOutsideActiveTimeBWP-Id-r16 | 0 |  |  |
| dormancyGroupOutsideActiveTime-r16 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.12.4.1.3.3-6: BWP-Downlink (Table 7.1.1.12.4.1.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Downlink ::= SEQUENCE { |  |  |  |
| bwp-Common SEQUENCE { |  |  |  |
| pdcch-ConfigCommon | Not present |  |  |
| } |  |  |  |
| bwp-Dedicated SEQUENCE { |  |  |  |
| pdcch-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| controlResourceSetToAddModList SEQUENCE(SIZE (1..3)) OF ControlResourceSet { | 1 entry |  |  |
| ControlResourceSet[1] | ControlResourceSet | TS 38.508-1 default value |  |
| } |  |  |  |
| controlResourceSetToReleaseList | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

###### 7.1.1.12.4.2	DRX adaptation / SCell dormancy indication / Intra-band non Contiguous CA

The scope and description of the present TC is the same as test case 7.1.1.12.4.1 with the following differences:

-	CA configuration: Intra-band non-Contiguous CA replaces Intra-band Contiguous CA

###### 7.1.1.12.4.3	DRX adaptation / SCell dormancy indication / Inter-band CA

The scope and description of the present TC is the same as test case 7.1.1.12.4.1 with the following differences:

-	CA configuration: Inter-band CA replaces Intra-band Contiguous CA

-	Cells configuration: NR Cell 10 replaces NR Cell 3

##### 7.1.1.12.5	PDCCH monitoring adaptation / Search Space Set Group Switching

7.1.1.12.5.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives searchSpaceGroupIdList-r17 in pdcch-Config in a RRC message }

then { UE starts monitoring PDCCH according to search space sets with group index 0 }

}

(2)

with { UE in RRC_CONNECTED state and not configured with searchSpaceSwitchDelay-r17 }

ensure that {

when { UE detects DCI format 1_1 with PDCCH monitoring adaptation field set to search space group ID 1 }

then { UE starts monitoring PDCCH as per search space sets with group index 1 after Pswitch symbols delay as configured per scs based on the support of UE processing capability 1 or 2 }

}

(3)

with { UE in RRC_CONNECTED state and configured with searchSpaceSwitchDelay-r17 }

ensure that {

when { UE detects DCI format 1_1 with PDCCH monitoring adaptation field set to search space set group ID 1 }

then { UE starts monitoring PDCCH as per search space sets with group index 1 after searchSpaceSwitchDelay-r17 symbols delay from the last symbol of PDCCH carrying DCI 1_1 }

}

(4)

with { UE supporting sssg-Switching-2BitInd-r17 and configured with searchSpaceSwitchDelay-r17 in RRC_CONNECTED state }

ensure that {

when { UE detects DCI format 1_1 with 2-bit PDCCH monitoring adaptation field set to search space set group index 2 }

then { UE starts monitoring PDCCH as per search space sets with group index 2 after searchSpaceSwitchDelay-r17 symbols delay from the last symbol of PDCCH carrying DCI 1_1 }

}

(5)

with { UE in RRC_CONNECTED and searchSpaceSwitchTimer-r17 is running and monitoring group index 1 }

ensure that {

when { searchSpaceSwitchTimer-r17 expires }

then { the UE continues monitoring PDCCH as per search space sets with group index 1 for Pswitch symbols based on the support of UE processing capability 1 or 2 }

}

(6)

with { UE in RRC_CONNECTED }

ensure that {

when { searchSpaceGroupIdList-r17 is removed in RRC message }

then { the UE starts monitoring PDCCH in all configured search space sets }

}

(7)

with { UE in RRC_CONNECTED state }

ensure that {

when { searchSpaceSwitchTimer-r17 expires }

then { the UE starts monitoring PDCCH as per search space sets with group index 0 after searchSpaceSwitchDelay-r17 symbols }

}

7.1.1.12.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.212, clause 7.3.1.2.2 and TS 38.213, clause 10.4. Unless otherwise stated these are Rel-17 requirements.

[TS 38.212, clause 7.3.1.2.2]

PDCCH monitoring adaptation indication – 0, 1 or 2 bits

…

-	1 or 2 bits, if pdcch-SkippingDurationList is not configured and if searchSpaceGroupIdList-r17 is configured

-	1 bit if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0 and search space set(s) with group index 1, and if the UE is not configured by searchSpaceGroupIdList-r17 with any search space set with group index 2;

-	2 bits if the UE is configured by searchSpaceGroupIdList-r17 with search space set(s) with group index 0, search space set(s) with group index 1 and search space set(s) with group index 2;

-	2 bits, if pdcch-SkippingDurationList is configured and if searchSpaceGroupIdList-r17 is configured

-	0 bit, otherwise

[TS 38.213, clause 10.4]

A UE can be provided

-	a group index for a respective Type3-PDCCH CSS set or USS set by searchSpaceGroupIdList for PDCCH monitoring on a serving cell,

-	a group index for a respective Type3-PDCCH CSS set or USS set by searchSpaceGroupIdList-r17 for PDCCH monitoring on an active DL BWP of a serving cell.

If the UE is not provided searchSpaceGroupIdList or searchSpaceGroupIdList-r17 for a search space set, the following procedures that are based on search space set group switching are not applicable for PDCCH monitoring according to the search space set.

…

When a UE is provided searchSpaceGroupIdList or searchSpaceGroupIdList-r17, the UE resets PDCCH monitoring according to search space sets with group index 0, if provided by searchSpaceGroupIdList or searchSpaceGroupIdList-r17.

A UE can be provided by searchSpaceSwitchDelay or searchSpaceSwitchDelay-r17 a number of symbols Pswitch where a minimum value of Pswitch is provided in Table 10.4-1 for UE processing capability 1 and UE processing capability 2 and SCS configuration μ. UE processing capability 1 for SCS configuration μ applies unless the UE indicates support for UE processing capability 2.

Table 10.4-1: Minimum value of Pswitch [symbols]

| μ | Minimum Pswitch value for UE processing capability 1 [symbols] | Minimum Pswitch value for UE processing capability 2 [symbols] |
| --- | --- | --- |
| 0 | 25 | 10 |
| 1 | 25 | 12 |
| 2 | 25 | 22 |
| 3 | 40 | - |
| 5 | 160 | - |
| 6 | 320 | - |

A UE can be provided, by searchSpaceSwitchTimer, a timer value for a serving cell that the UE is provided searchSpaceGroupIdList or, if provided, for a set of serving cells provided by cellGroupsForSwitchList. The UE decrements the timer value by one after each slot based on a reference SCS configuration that is the smallest SCS configuration μ among all configured DL BWPs in the serving cell, or in the set of serving cells. The UE maintains the reference SCS configuration during the timer decrement procedure.

…

A UE can be provided group indexes for a Type3-PDCCH CSS set or USS set by searchSpaceGroupIdList-r17 for PDCCH monitoring on an active DL BWP of a serving cell and, if the UE is not provided pdcch-SkippingDurationList for the active DL BWP of the serving cell, a DCI format 0_1 and a DCI format 0_2 that schedule PUSCH transmissions and a DCI format 1_1 and a DCI format 1_2 that schedule PDSCH receptions can include a PDCCH monitoring adaptation field of 1 bit or of 2 bits for the serving cell.

If the field has 1 bit and for PDCCH monitoring by the UE according to Type3-PDCCH CSS sets or USS sets on the active DL BWP of the serving cell

-	a '0' value for the bit indicates start of PDCCH monitoring according to search space sets with group index 0 and stop of PDCCH monitoring according to search space sets with other group indexes, if any

-	a '1' value for the bit indicates start of PDCCH monitoring according to search space sets with group index 1 and stop of PDCCH monitoring according to search space sets with other group indexes, if any, and the UE sets the timer value to the one provided by searchSpaceSwitchTimer-r17, if provided

If the field has 2 bits and for PDCCH monitoring by the UE according to Type3-PDCCH CSS sets or USS sets on the active DL BWP of the serving cell

-	a '00' value for the bit indicates start of PDCCH monitoring according to search space sets with group index 0 and stop of PDCCH monitoring according to search space sets with other group indexes, if any

-	a '01' value for the bit indicates start of PDCCH monitoring according to search space sets with group index 1 and stop of PDCCH monitoring according to search space sets with other group indexes, if any, and the UE sets the timer value to the one provided by searchSpaceSwitchTimer-r17, if provided

-	a '10' value for the bit indicates start of PDCCH monitoring according to search space sets with group index 2 and stop of PDCCH monitoring according to search space sets with other group indexes, if any, and the UE sets the timer value to the one provided by searchSpaceSwitchTimer-r17, if provided

-	a '11' value is reserved

A UE can be provided a set of durations by pdcch-SkippingDurationList and group indexes for a Type3-PDCCH CSS set or USS set by searchSpaceGroupIdList-r17 for PDCCH monitoring on an active DL BWP of a serving cell and, a DCI format 0_1 and a DCI format 0_2 that schedule PUSCH transmissions, and a DCI format 1_1 and a DCI format 1_2 that schedule PDSCH receptions can include a PDCCH monitoring adaptation field of 2 bits.

If the set of durations includes one value and for PDCCH monitoring by the UE according to Type3-PDCCH CSS sets or USS sets on the active DL BWP of the serving cell

-	a '00' value for the bits indicates start of PDCCH monitoring according to search space sets with group index 0 and stop of PDCCH monitoring according to search space sets with group index 1, if any

-	a '01' value for the bits indicates start of PDCCH monitoring according to search space sets with group index 1 and stop of PDCCH monitoring according to search space sets with group index 0, if any, and the UE sets the timer value to the one provided by searchSpaceSwitchTimer-r17, if provided

-	a '10' value for the bits indicates skipping PDCCH monitoring for a duration provided by the value in the set of durations

-	a '11' value is reserved

If the set of durations includes two values and for PDCCH monitoring by the UE according to Type3-PDCCH CSS sets or USS sets on active DL BWP of the serving cell

-	a '00' value for the bits indicates start of PDCCH monitoring according to search space sets with group index 0 and stop of PDCCH monitoring according to search space sets with group index 1, if any

-	a '01' value for the bits indicates start of PDCCH monitoring according to search space sets with group index 1 and stop of PDCCH monitoring according to search space sets with group index 0, if any, and the UE sets the timer value to the one provided by searchSpaceSwitchTimer-r17, if provided

-	a '10' value for the bits indicates skipping PDCCH monitoring for a duration provided by the first value in the set of durations

-	a '11' value for the bits indicates skipping PDCCH monitoring for a duration provided by the second value in the set of durations

When the PDCCH monitoring adaptation field indicates to a UE to start PDCCH monitoring according to search space sets with a first group index and stop PDCCH monitoring according to search space sets with a second group index, the UE applies the indication

-	at the beginning of a first slot that is at least Pswitch symbols after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field when μ∈0, 1, 2, 3,

-	at the beginning of a first slot, of a slot group of Xs slots, that is at least Pswitch symbols after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field when μ∈5, 6

…

If the UE changes to a new active DL BWP of the serving cell by the expiration of bwp-InactivityTimer, or by RA procedure requested by higher layers [11, TS 38.321], or by RRC configuration, the UE

-	resumes PDCCH monitoring according to the search space sets on the new active BWP of the serving cell when UE is in a PDCCH skipping duration, if the UE is not provided searchSpaceGroupIdList-r17 on the new active DL BWP

-	monitors PDCCH according to search space sets with group index 0 on the new active BWP of the serving cell, if the UE is provided searchSpaceGroupIdList-r17.

If a UE is provided group indexes for a Type3-PDCCH CSS set or a USS set by searchSpaceGroupIdList-r17 and a timer value by searchSpaceSwitchTimer-r17 for PDCCH monitoring an active DL BWP of on a serving cell and the timer is running, the UE

-	resets the timer after a slot of the active DL BWP of the serving cell if the UE detects a DCI format in a PDCCH reception in the slot for with CRC scrambled by C-RNTI/CS-RNTI/MCS-C-RNTI/G-RNTI for multicast/G-CS-RNTI

-	otherwise, decrements the timer value by one after a slot of the active DL BWP of the serving cell

When the timer expires in a first slot, the UE monitors PDCCH on the serving cell according to search space sets with group index 0 starting in a second slot that

-	is not earlier than Pswitch symbols after the first slot when μ∈0, 1, 2, 3,

-	is a first slot in a slot group of Xs slots that is not earlier than Pswitch symbols after the first slot when μ∈5, 6,

-	is not earlier than a slot where a PDCCH skipping duration expires, if applicable

When a UE receives

-	a first PDCCH in a first slot that provides a DCI format with a PDCCH monitoring adaptation field having a first value indicating skipping PDCCH monitoring, or indicating start of PDCCH monitoring according to a search space sets with a first group index and stop of PDCCH monitoring according to search space sets with a second group index, for an active DL BWP and

-	a second PDCCH that provides a DCI format with a PDCCH monitoring adaptation field having a second value indicating skipping PDCCH monitoring, or indicating start of PDCCH monitoring according to search space sets with a first group index and stop of PDCCH monitoring according to search space sets with a second group index different than the first group index, for the active DL BWP where the second PDCCH is received

-	in the first slot if the first value indicates skipping PDCCH monitoring

-	before a slot that is at least Pswitch symbols after the first slot if the first value indicates start of PDCCH monitoring according to search space sets with a first group index

the UE does not expect the second value to be different than the first value.

A UE does not expect to receive in a second slot a PDCCH on an active DL BWP that provides a DCI format indicating skipping PDCCH monitoring, or start of PDCCH monitoring according to search space sets with group index 1 or 2 for the active DL BWP, if the second slot is not at least Pswitch symbols after a first slot where the timer expires.

7.1.1.12.5.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that set to return no data in uplink.

7.1.1.12.5.3.2	Test procedure sequence

Table 7.1.1.12.5.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits NR RRCReconfiguration message to configure search space set group as per Table 7.1.1.12.5.3.3-1. (Note 1) | <-- | (NR RRC: RRCReconfiguration) | - | - |
| 2 | The UE transmits NR RRCReconfigurationComplete message. (Note 2) | --> | (NR RRC: RRCReconfigurationComplete) | - | - |
| 3 | SS transmits a downlink assignment in search space ID 2 corresponding to group ID 0 addressed to C-RNTI with PDCCH monitoring adaptation field indicating group ID 0. | <-- | DCI format 1_1 | - | - |
| 4 | SS transmits in the indicated downlink assignment a MAC PDU. | <-- | MAC PDU | - | - |
| 5 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 4? | --> | HARQ ACK | 1 | P |
| 6 | SS transmits a downlink assignment in search space ID 2 addressed to C-RNTI with PDCCH monitoring adaptation field indicating group ID 1. | <-- | DCI format 1_1 | - | - |
| 7 | SS transmits in the indicated downlink assignment a MAC PDU. | <-- | MAC PDU | - | - |
| 8 | The UE transmits a HARQ ACK for the DL MAC PDU in Step 7. | --> | HARQ ACK | - | - |
| 9 | In a PDCCH occasion of next available slot after Pswitch symbols from step 6, SS transmits a downlink assignment in search space ID 4 corresponding to group ID 1 addressed to C-RNTI with PDCCH monitoring adaptation field indicating group ID 1. (Note 3) | <-- | DCI format 1_1 | - | - |
| 10 | SS transmits in the indicated downlink assignment a MAC PDU. | <-- | MAC PDU | - | - |
| 11 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 10? | --> | HARQ ACK | 2 | P |
| 12 | SS waits for searchSpaceSwitchTimer-r17 expiry. | - | - | - | - |
| 13 | In a PDCCH occasion with in Pswitch symbols after searchSpaceSwitchTimer-r17 expiry, SS transmits a downlink assignment in search space ID 4 corresponding to group ID 1 addressed to C-RNTI with PDCCH monitoring adaptation field indicating group ID 0. | <-- | DCI format 1_1 | - | - |
| 14 | SS transmits in the indicated downlink assignment a MAC PDU. | <-- | MAC PDU | - | - |
| 15 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 14? | --> | HARQ ACK | 5 | P |
| 16 | SS transmits NR RRCReconfiguration message to configure search space set group as per Table 7.1.1.12.5.3.3-1. (Note 1) | <-- | (NR RRC: RRCReconfiguration) | - | - |
| 17 | The UE transmits NR RRCReconfigurationComplete message. (Note 2) | --> | (NR RRC: RRCReconfigurationComplete) |  | - |
| - | EXCEPTION: Steps 18a1 to 18b7 describe behavior that depends on UE capability. | - | - | - | - |
| 18a1 | IF pc_sssg_Switching_2BitInd_r17 THEN SS transmits a downlink assignment in search space ID 2 addressed to C-RNTI with PDCCH monitoring adaptation field indicating group ID 2. | <-- | DCI format 1_1 | - | - |
| 18a2 | SS transmits in the indicated downlink assignment a MAC PDU. | <-- | MAC PDU | - | - |
| 18a3 | The UE transmits a HARQ ACK for the DL MAC PDU in Step 18a2. | --> | HARQ ACK | - | - |
| 18a4 | In a PDCCH occasion of next available slot after searchSpaceSwitchDelay-r17 symbols from step 18a1, SS transmits a downlink assignment in search space ID 5 corresponding to group ID 2 addressed to C-RNTI with PDCCH monitoring adaptation field indicating group ID 2. | <-- | DCI format 1_1 | - | - |
| 18a5 | SS transmits in the indicated downlink assignment a MAC PDU. | <-- | MAC PDU | - | - |
| 18a6 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 18a5? | --> | HARQ ACK | 4 | P |
| 18a7 | SS waits for searchSpaceSwitchTimer-r17 expiry started at step 18a4. | - | - | - | - |
| 18b1 | IF NOT pc_sssg_Switching_2BitInd_r17 THEN SS transmits a downlink assignment in search space ID 2 addressed to C-RNTI with PDCCH monitoring adaptation field indicating group ID 1. | <-- | DCI format 1_1 | - | - |
| 18b2 | SS transmits in the indicated downlink assignment a MAC PDU. | <-- | MAC PDU | - | - |
| 18b3 | The UE transmits a HARQ ACK for the DL MAC PDU in Step 18b2. | --> | HARQ ACK | - | - |
| 18b4 | In a PDCCH occasion of next available slot after searchSpaceSwitchDelay-r17 symbols from step 18b1, SS transmits a downlink assignment in search space ID 4 corresponding to group ID 1 addressed to C-RNTI with PDCCH monitoring adaptation field indicating group ID 1. | <-- | DCI format 1_1 | - | - |
| 18b5 | SS transmits in the indicated downlink assignment a MAC PDU. | <-- | MAC PDU | - | - |
| 18b6 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 18b5? | --> | HARQ ACK | 3 | P |
| 18b7 | SS waits for searchSpaceSwitchTimer-r17 expiry started at step 18b4. | - | - | - | - |
| 18A | SS waits for searchSpaceSwitchDelay-r17 expiry after step 18a7/18b7. | - | - | - | - |
| 18B | In a PDCCH occasion of next available slot, SS transmits a downlink assignment in search space ID 2 corresponding to group ID 0 addressed to C-RNTI with PDCCH monitoring adaptation field indicating group ID 0. | <-- | DCI format 1_1 | - | - |
| 18C | SS transmits in the indicated downlink assignment a MAC PDU. | <-- | MAC PDU | - | - |
| 18D | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 18B? | --> | HARQ ACK | 7 | P |
| 19 | SS transmits NR RRCReconfiguration message excluding search space set group configuration. (Note 1) | <-- | NR RRC: RRCReconfiguration | - | - |
| 20 | The UE transmits NR RRCReconfigurationComplete message. (Note 2) | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 21 | SS transmits a downlink assignment on search space ID 5 addressed to C-RNTI. | <-- | DCI format 1_1 | - | - |
| 22 | SS transmits in the indicated downlink assignment a MAC PDU. | <-- | MAC PDU | - | - |
| 23 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 22? | --> | HARQ ACK | 6 | P |
| Note 1: For EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration. Note 2: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. Note 3: IF (pc_searchSpaceSetGrp_switchCap2_r17 and FR1) THEN Pswitch is derived from TS 38.213 Table 10.4-1 for processing capability 2 as per configured scs μ ∈ {0,1,2} ELSE Pswitch is derived from TS 38.213 Table 10.4-1 for processing capability 1 as per configured scs μ ∈ {0,1,2,3}. |  |  |  |  |  |

7.1.1.12.5.3.3	Specific message contents

Table 7.1.1.12.5.3.3-1: RRCReconfiguration (step 1, step 16, step 19, Table 7.1.1.12.5.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| secondaryCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC |
|  | Not present |  | NR |
| nonCriticalExtension | Not present |  | EN-DC |
| nonCriticalExtension SEQUENCE{ |  |  | NR |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.12.5.3.3-2: CellGroupConfig (Table 7.1.1.12.5.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated | ServingCellConfig |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.12.5.3.3-3: ServingCellConfig (Table 7.1.1.12.5.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkDedicated |  |  |
| } |  |  |  |

Table 7.1.1.12.5.3.3-3A: BWP-DownlinkDedicated (Table 7.1.1.12.5.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkDedicated ::= SEQUENCE { |  |  |  |
| pdcch-ServingCellConfig CHOICE { |  |  |  |
| setup | PDCCH-Config |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.12.5.3.3-4: PDCCH-Config (Table 7.1.1.12.5.3.3-3A)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-95 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-Config ::= SEQUENCE { |  |  |  |
| searchSpacesToAddModList SEQUENCE(SIZE (1..10)) OF SearchSpace { | 3 entries |  |  |
| SearchSpace[1] | SearchSpace with condition USS1 | Entry 1 |  |
| SearchSpace[2] | SearchSpace with condition USS2 | Entry 2 |  |
| SearchSpace[3] | SearchSpace with condition USS3 | Entry 3 |  |
| } |  |  |  |
| searchSpacesToAddModListExt-v1700 | Not present |  | Step 19 |
| searchSpacesToAddModListExt-v1700 SEQUENCE(SIZE (1..10)) OF SearchSpaceExt-v1700 { | 3 entries if pc_sssg_Switching_2BitInd_r17, 2 entries otherwise |  | Step 1, Step 16 |
| SearchSpaceExt-v1700[1] SEQUENCE { | 1 entry | Entry 1 |  |
| searchSpaceGroupIdList-r17[0] | 0 | Entry 1 |  |
| } |  |  |  |
| SearchSpaceExt-v1700[2] SEQUENCE { | 1 entry | Entry 2 |  |
| searchSpaceGroupIdList-r17[0] | 1 | Entry 1 |  |
| } |  |  |  |
| SearchSpaceExt-v1700[3] SEQUENCE { | 1 entry | Entry 3 |  |
| searchSpaceGroupIdList-r17[0] | 2 | Entry 1 |  |
| } |  |  |  |
| searchSpaceSwitchConfig-r17 | Not present |  | Step 19 |
| searchSpaceSwitchConfig-r17 ::= SEQUENCE { |  |  | Step 1, Step 16 |
| searchSpaceSwitchTimer-r17 | 19 |  |  |
| searchSpaceSwitchDelay-r17 | 50 |  | Step 16 |
|  | Not present |  | Step 1 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.12.5.3.3-5: SearchSpace (Table 7.1.1.12.5.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-162 with condition USS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 2 |  | USS1 |
|  | 4 |  | USS2 |
|  | 5 |  | USS3 |
| controlResourceSetId | 1 |  |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl5 | 1 |  | USS1 |
| sl5 | 2 |  | USS2 |
| sl10 | 5 |  | USS3 |
| } |  |  |  |
| duration | Not present |  |  |
| monitoringSymbolsWithinSlot | 10000000000000 |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n1 |  |  |
| aggregationLevel4 | n0 |  |  |
| aggregationLevel8 | n0 |  |  |
| aggregationLevel16 | n0 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| USS1 | Configuration of USS1 |
| USS2 | Configuration of USS2 |
| USS3 | Configuration of USS3 |

Table 7.1.1.12.5.3.3-6: Physical layer parameters for DCI format 1_1 (All steps, Table 7.1.1.12.5.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.3.6.1.2.2-1 with condition SSSG |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| PDSCH-to-HARQ_feedback timing indicator | Corresponding to K1=7 slots as per dl-DataToUL-ACK in Table 4.6.3-112 TS 38.508-1 [4]. | “101”B |  |
|  | Corresponding to K1=8 slots as per dl-DataToUL-ACK in Table 4.6.3-112 TS 38.508-1 [4]. | “110”B | TDD AND SCS15 AND USS1 |
|  | Corresponding to K1=9 slots as per dl-DataToUL-ACK in Table 4.6.3-112 TS 38.508-1 [4]. | “111”B | TDD AND SCS15 AND USS3 |
|  | Corresponding to K1=7 or 3 slots depending on the transmission occasion and as per dl-DataToUL-ACK in Table 4.6.3-112 TS 38.508-1 [4]. | “101”B/”001”B | TDD AND SCS30 AND USS1 |
|  | Corresponding to K1=6 or 2 slots depending on the transmission occasion and as per dl-DataToUL-ACK in Table 4.6.3-112 TS 38.508-1 [4]. | “100”B/”000”B | TDD AND SCS30 AND USS2 |
|  | Corresponding to K1=4 slots as per dl-DataToUL-ACK in Table 4.6.3-112 TS 38.508-1 [4]. | “001”B | TDD AND SCS30 AND USS3 |
|  | Corresponding to K1=8 slots as per dl-DataToUL-ACK in Table 4.6.3-112 TS 38.508-1 [4]. | “110”B | TDD AND SCS120 AND SS1 |
|  | Corresponding to K1=7 slots as per dl-DataToUL-ACK in Table 4.6.3-112 TS 38.508-1 [4]. | “101”B | TDD AND SCS120 AND SS2 |
|  | Corresponding to K1=4 slots as per dl-DataToUL-ACK in Table 4.6.3-112 TS 38.508-1 [4]. | “010”B | TDD AND SCS120 AND SS3 |

| Condition | Explanation |
| --- | --- |
| USS1 | Configuration of USS1 |
| USS2 | Configuration of USS2 |
| USS3 | Configuration of USS3 |

#### 7.1.1.13	Small Data Transmission (SDT)

##### 7.1.1.13.1	RA Based SDT / 2-step RACH / Successful

7.1.1.13.1.1	Test Purpose (TP)

(1)

with { UE in NR RRC_INACTIVE state and SDT-CG-Config-r17 is not configured and Random Access resources for 2-step RA-SDT are configured }

ensure that {

when { UE has small data to transmit and the data volume of the pending UL data across all RBs configured for SDT is less than or equal to sdt-DataVolumeThreshold and RSRP is above the configured sdt-RSRP-Threshold }

then { UE shall initiate 2-step RA based SDT procedure }

}

(2)

with { UE in NR RRC_INACTIVE state and SDT-CG-Config-r17 is not configured and Random Access resources for 2-step RA-SDT are configured }

ensure that {

when { UE has small data to transmit and the data volume of the pending UL data across all RBs configured for SDT is greater than sdt-DataVolumeThreshold and RSRP is above the configured sdt-RSRP-Threshold }

then { UE shall not initiate RA based SDT procedure and starts normal RRC Resume procedure }

}

(3)

with { UE in NR RRC_INACTIVE state and SDT-CG-Config-r17 is not configured and Random Access resources for RA-SDT are configured }

ensure that {

when { UE initiates RA based SDT procedure }

then { UE is successfully able to send and receive subsequent SDT data }

}

(4)

with { UE in NR RRC_INACTIVE state and SDT-CG-Config-r17 is not configured and Random Access resources for 2-step RA-SDT are configured }

ensure that {

when { UE has small data to transmit and the data volume of the pending UL data across all RBs configured for SDT is less than or equal sdt-DataVolumeThreshold and RSRP is below the configured sdt-RSRP-Threshold }

then { UE shall not initiate RA based SDT procedure and starts normal RRC Resume procedure }

}

7.1.1.13.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 38.321, clause 5.1.1b, 5.1.1c and 5.27. Unless otherwise stated these are Rel-17 requirements.

[TS 38.321, clause 5.1.1b]

The MAC entity shall:

1>	if the BWP selected for Random Access procedure is configured with both set(s) of Random Access resources with MSG3 repetition indication and set(s) of Random Access resources without MSG3 repetition indication and the RSRP of the downlink pathloss reference is less than rsrp-ThresholdMsg3; or

1>	if the BWP selected for Random Access procedure is only configured with the set(s) of Random Access resources with MSG3 repetition indication:

2>	assume MSG3 repetition is applicable for the current Random Access procedure.

1>	else:

2>	assume MSG3 repetition is not applicable for the current Random Access procedure.

NOTE 1:	Void.

1>	if contention-free Random Access Resources have not been provided for this Random Access procedure and one or more of the features including RedCap and/or a specific NSAG(s) and/or SDT and/or MSG3 repetition is applicable for this Random Access procedure:

NOTE 2: The applicability of SDT is determined by MAC entity according to clause 5.27. The applicability of specific NSAG(s) is determined by upper layers when the Random Access procedure is initiated. The applicability of RedCap is also determined by upper layers when Random Access procedure is initiated and it is applicable to the Random Access procedures initiated by PDCCH orders and any Random Access procedure initiated by the MAC entity.

2>	if none of the sets of Random Access resources are available for any feature applicable to the current Random Access procedure (as specified in clause 5.1.1c):

3>	select the set(s) of Random Access resources that are not associated with any feature indication (as specified in clause 5.1.1c) for this Random Access procedure.

2>	else if there is one set of Random Access resources available which can be used for indicating all features triggering this Random Access procedure:

3>	select this set of Random Access resources for this Random Access procedure.

2>	else (i.e. there are one or more sets of Random Access resources available that are configured with indication(s) for a subset of all features triggering this Random Access procedure):

3>	select a set of Random Access resources from the available set(s) of Random Access resources based on the priority order indicated by upper layers as specified in clause 5.1.1d for this Random Access Procedure.

1>	else if contention-free Random Access Resources have been provided for this Random Access procedure and RedCap is applicable for the current Random Access procedure and there is one set of Random Access resources available that is only configured with RedCap indication:

2>	select this set of Random Access resources for this Random Access procedure.

1>	else:

2>	select the set of Random Access resources that are not associated with any feature indication (as specified in clause 5.1.1c) for the current Random Access procedure.

[TS 38.321, clause 5.1.1c]

The MAC entity shall for each set of configured Random Access resources for 4-step RA type and for each set of configured Random Access resources for 2-step RA type:

1>	if redCap is set to true for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for a Random Access procedure for which RedCap is not applicable.

1>	if smallData is set to true for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for the Random Access procedure which is not triggered for RA-SDT.

1>	if NSAG-List is configured for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for the Random Access procedure unless it is triggered for any one of the NSAG-ID(s) in the NSAG-List.

1>	if msg3-Repetitions is set to true for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for the Random Access procedure if Msg3 repetition is not applicable.

1>	if a set of Random Access resources is not configured with FeatureCombination:

2>	consider the set of Random Access resources to not associated with any feature.

[TS 38.321, clause 5.27]

The MAC entity may be configured by RRC with SDT and the SDT procedure may be initiated by RRC layer. The SDT procedure can be performed either by Random Access procedure with 2-step RA type or 4-step RA type (i.e., RA-SDT) or by configured grant Type 1 (i.e., CG-SDT).…

If RA-SDT is selected above and after the Random Access procedure is successfully completed (see clause 5.1.6), the UE monitors PDCCH addressed to C-RNTI received in random access response until the RA-SDT procedure is terminated. If CG-SDT is selected above and after the initial transmission for CG-SDT is performed, the UE monitors PDCCH addressed to C-RNTI as stored in UE Inactive AS context as specified in TS 38.331 [5] and CS-RNTI until the CG-SDT procedure is terminated.

7.1.1.13.1.3	Test description

7.1.1.13.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

UE:

-	None.

Preamble:

-	The UE is in state 3N-A and Test Mode Activated according to TS 38.508-1 [4] Table 4.4A.2-3 with UE test loop mode B is established with IP PDU delay set to 6 seconds. IF pc_logicalChannelSR_DelayTimer the DRB is configured according to Table 7.1.1.13.1.3.1-1 for SDT operation.

Table 7.1.1.13.1.3.1-1: Logical Channel Configuration Settings

| Parameter | SDT DRB |
| --- | --- |
| logicalChannelSR-DelayTimerApplied | True |
| logicalChannelSR-DelayTimer | sf512 |

7.1.1.13.1.3.2	Test procedure sequence

Table 7.1.1.13.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits a downlink assignment including the C-RNTI assigned to the UE | <-- | (PDCCH (C-RNTI)) | - | - |
| 2 | SS transmits in the indicated downlink assignment a RLC PDU in a MAC PDU on the DRB configured with SDT (SDT Data > sdt-DataVolumeThreshold). | <-- | MAC PDU | - | - |
| 3 | The SS transmits an RRCRelease message including sdt-Config-r17 in suspendConfig. | <-- | NR RRC: RRCRelease | - | - |
| 4 | Check: Does UE transmit MSGA using preamble on PRACH after IP PDU Delay expires? | --> | MAC PDU (including NR RRC: RRCResumeRequest ) | 2 | P |
| 5 | The SS transmits a MSGB including a successRAR MAC subPDU containing matching Contention Resolution Identity, C-RNTI and Timing Advance Command. | <-- | MAC PDU (successRAR) | - | - |
| 6 | The SS transmits an RRCResume message. | <-- | NR RRC: RRCResume | - | - |
| - | EXCEPTION: Steps 7 and 8 can happen in any order | - | - | - | - |
| 7 | The UE transmits an RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 8 | Check: Does the UE transmit a MAC PDU containing Loop backed PDU? | --> | MAC PDU (containing 1 MAC sub PDU containing RLC SDU) | 2 | P |
| 9 | The SS transmits an OPEN UE TEST LOOP message. | <-- | TC: OPEN UE TEST LOOP | - | - |
| 10 | The UE transmits an OPEN UE TEST LOOP COMPLETE message. | --> | TC: OPEN UE TEST LOOP COMPLETE | - | - |
| 11 | SS transmits a CLOSE UE TEST LOOP message. | <-- | TC: CLOSE UE TEST LOOP | - | - |
| 12 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message. | --> | TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| 13 | SS transmits a downlink assignment including the C-RNTI assigned to the UE | <-- | (PDCCH (C-RNTI)) | - | - |
| 14 | SS transmits in the indicated downlink assignment a RLC PDU in a MAC PDU on the DRB configured with SDT (SDT Data < sdt-DataVolumeThreshold). | <-- | MAC PDU | - | - |
| 15 | The SS transmits an RRCRelease message including sdt-Config-r17 in suspendConfig. | <-- | NR RRC: RRCRelease | - | - |
| 16 | Check: Does UE transmit MSGA using preamble on PRACH and associated PUSCH resource containing RLC PDU on DRB with SDT configured after IP PDU Delay expires? | --> | MAC PDU (including NR RRC: RRCResumeRequest, RLC PDU on DRB with SDT configured) | 1 | P |
| 17 | The SS transmits a MSGB including a successRAR MAC subPDU containing matching Contention Resolution Identity, C-RNTI and Timing Advance Command. | <-- | MAC PDU (successRAR) | - | - |
| - | EXCEPTION: Steps 17a1 to 17a3 describe behaviour that depends on the UE capability. | - | - | - | - |
| 17a1 | IF pc_logicalChannelSR_DelayTimer THEN SS transmits in the indicated downlink assignment a RLC PDU in a MAC PDU on the DRB configured with SDT (SDT Data <= sdt-DataVolumeThreshold). | <-- | MAC PDU |  |  |
| 17a2 | SS transmits an UL Grant, allowing the UE to return the RLC SDU as received in step 17a1, on PDCCH with the C-RNTI assigned to the UE. | <-- | (UL Grant (C-RNTI)) | - | - |
| 17a3 | Check: Does the UE transmit a MAC PDU including one RLC SDU? | --> | MAC PDU | 3 | P |
| 17A | The SS transmits an RRCResume message to bring UE to RRC_CONNECTED State. | <-- | NR RRC: RRCResume | - | - |
| 17B | The UE transmits an RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 18 | The SS changes the parameter ‘sdt-RSRP-Threshold-r17’ in SIB1 of NR Cell 1 to 76 and starts broadcasting updated SIB1. Note: This value should result in meeting condition ‘RSRP is below the configured sdt-RSRP-Threshold’ | - | - | - | - |
| 19 | The SS transmits an OPEN UE TEST LOOP message. | <-- | TC: OPEN UE TEST LOOP | - | - |
| 20 | The UE transmits an OPEN UE TEST LOOP COMPLETE message. | --> | TC: OPEN UE TEST LOOP COMPLETE | - | - |
| 21 | SS transmits a CLOSE UE TEST LOOP message. | <-- | TC: CLOSE UE TEST LOOP | - | - |
| 22 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message. | --> | TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| 23 | SS transmits a downlink assignment including the C-RNTI assigned to the UE | <-- | (PDCCH (C-RNTI)) | - | - |
| 24 | SS transmits in the indicated downlink assignment a RLC PDU in a MAC PDU on the DRB configured with SDT (SDT Data < sdt-DataVolumeThreshold). | <-- | MAC PDU | - | - |
| 25 | The SS transmits an RRCRelease message including sdt-Config-r17 in suspendConfig. | <-- | NR RRC: RRCRelease | - | - |
| 26 | Check: Does UE transmit MSGA using preamble on PRACH after IP PDU Delay expires? | --> | MAC PDU (including NR RRC: RRCResumeRequest) | 4 | P |
| 27 | The SS transmits a MSGB including a successRAR MAC subPDU containing matching Contention Resolution Identity, C-RNTI and Timing Advance Command. | <-- | MAC PDU (successRAR) | - | - |
| 28 | The SS transmits an RRCResume message. | <-- | NR RRC: RRCResume | - | - |
| - | Exception: Steps 29 and 30 can happen in any order | - | - | - | - |
| 29 | The UE transmits an RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 30 | Check: Does the UE transmit a MAC PDU containing Loop backed PDU? | --> | MAC PDU (containing 1 MAC sub PDU containing RLC SDU) | 4 | P |
| 31 | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |

7.1.1.13.1.3.3	Specific message contents

Table 7.1.1.13.1.3.3-1: CLOSE UE TEST LOOP (Steps 11, 21 and preamble Table 7.1.1.13.1.3.2-1 )

| Derivation Path: TS 36.508-1 [7] table 4.7A-3 condition UE test loop mode B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE test loop mode B LB setup |  |  |  |
| IP PDU delay | '0000 0110'B | 6 seconds |  |  |

Table 7.1.1.13.1.3.3-2: SIB1 (preamble and step 18, Table 7.1.1.13.1.3.2-1)

| Derivation path: TS 38.508-1 [4] Table 4.6.1-28 with Condition SDT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| servingCellConfigCommon | ServingCellConfigCommon | Table 7.1.1.13.1.3.3-3 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sdt-ConfigCommon-r17 SEQUENCE { |  |  |  |
| sdt-RSRP-Threshold-r17 | 66 (-90dBm) |  | Preamble |
|  | 76 (-80dBm) |  | Step 18 |
| sdt-LogicalChannelSR-DelayTimer-r17 | sf512 |  |  |
| sdt-DataVolumeThreshold-r17 | byte32 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.1.3.3-3: ServingCellConfigCommon (Table 7.1.1.13.1.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| uplinkConfigCommon SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkCommon | Table 7.1.1.13.1.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.1.3.3-4: BWP-UplinkCommon (Table 7.1.1.13.1.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkCommon ::= SEQUENCE { |  |  |  |
| msgA-ConfigCommon-r16 CHOICE { |  |  |  |
| setup | MsgA-ConfigCommon | TS 38.508-1 [4] Table 4.6.3-81A |  |
| } |  |  |  |
| AdditionalRACH-ConfigList-r17 SEQUENCE { |  |  |  |
| AdditionalRACH-Config-r17 SEQUENCE { |  |  |  |
| rach-ConfigCommon-r17 | Not present |  |  |
| msgA-ConfigCommon-r17 | MsgA-ConfigCommon-r16 | Table 7.1.1.13.1.3.3-5 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.1.3.3-5: MsgA-ConfigCommon-r16 (Table 7.1.1.13.1.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-81A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MsgA-ConfigCommon-r16 :: = SEQUENCE { |  |  |  |
| rach-ConfigCommonTwoStepRA-r16 | RACH-ConfigCommonTwoStepRA-r16 | Table 7.1.1.13.1.3.3-6 |  |
| } |  |  |  |

Table 7.1.1.13.1.3.3-6: RACH-ConfigCommonTwoStepRA-r16 (Table 7.1.1.13.1.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-128A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommonTwoStepRA-r16 ::= SEQUENCE { |  |  |  |
| featureCombinationPreamblesList-r17 SEQUENCE (SIZE(1..maxFeatureCombPreamblesPerRACHResource-r17)) OF FeatureCombinationPreambles-r17 { |  |  |  |
| FeatureCombinationPreambles-r17 | FeatureCombinationPreambles | Table 7.1.1.13.1.3.3-7 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.1.3.3-7: FeatureCombinationPreambles (Table 7.1.1.13.1.3.3-6)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-56E |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FeatureCombinationPreambles-r17 ::= SEQUENCE { |  |  |  |
| featureCombination-r17 ::= SEQUENCE { |  |  |  |
| smallData-r17 | true |  |  |
| } |  |  |  |
| startPreambleForThisPartition-r17 | 8 | Randomly selected |  |
| numberOfPreamblesPerSSB-ForThisPartition-r17 | 12 |  |  |
| ssb-SharedRO-MaskIndex-r17 | Not present |  |  |
| groupBconfigured-r17 | Not present |  |  |
| separateMsgA-PUSCH-Config-r17 | MsgA-PUSCH-Config | Table 7.1.1.13.1.3.3-8 |  |
| msgA-RSRP-Threshold-r17 | 57 | -100 dBm |  |
| rsrp-ThresholdSSB-r17 | RSRP-Range | TS 38.508-1 [4] table 4.6.3-152 |  |
| deltaPreamble-r17 | Not present |  |  |
| } |  |  |  |

Table 7.1.1.13.1.3.3-8: MsgA-PUSCH-Config (Table 7.1.1.13.1.3.3-7)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-81B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MsgA-PUSCH-Config-r16 ::= SEQUENCE { |  |  |  |
| msgA-PUSCH-ResourceGroupA-r16 SEQUENCE { |  |  |  |
| msgA-MCS-r16 | 1 |  |  |
| nrofPRBs-PerMsgA-PO-r16 | 15 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.1.3.3-9: RRCRelease (Steps 3, 15 and 25 Table 7.1.1.13.2.3.2-1)

| Derivation Path: TS 38.508-1 [4] table 4.6.1-16 with condition NR_RRC_INACTIVE and SDT |  |  |  |
| --- | --- | --- | --- |
| RRCRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |
| suspendConfig SEQUENCE { |  |  |  |
| sdt-Config-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sdt-DRB-List-r17 SEQUENCE (SIZE (0..maxDRB)) OF DRB-Identity { | 1 entry |  |  |
| DRB-Identity[1] | DRB-Identity using condition DRBj | Entry 1 j is the ID of the DRB established during the preamble which is allocated according to internal TTCN mapping |  |
| } |  |  |  |
| sdt-SRB2-Indication-r17 | Not present |  |  |
| sdt-MAC-PHY-CG-Config-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 7.1.1.13.2	RA Based SDT / 4-step RACH / Successful

7.1.1.13.2.1	Test Purpose (TP)

(1)

with { UE in NR RRC_INACTIVE state and SDT-CG-Config-r17 is not configured and Random Access resources for RA-SDT are configured }

ensure that {

when { UE has small data to transmit and the data volume of the pending UL data across all RBs configured for SDT is less than or equal to sdt-DataVolumeThreshold and RSRP is above the configured sdt-RSRP-Threshold }

then { UE shall initiate 4-step RA based SDT procedure }

}

(2)

with { UE in NR RRC_INACTIVE state and SDT-CG-Config-r17 is not configured and Random Access resources for RA-SDT are configured }

ensure that {

when { UE has small data to transmit and the data volume of the pending UL data across all RBs configured for SDT is greater than sdt-DataVolumeThreshold and RSRP is above the configured sdt-RSRP-Threshold }

then { UE shall not initiate RA based SDT procedure and starts normal RRC Resume procedure }

}

(3)

with { UE in NR RRC_INACTIVE state and SDT-CG-Config-r17 is not configured and Random Access resources for RA-SDT are configured }

ensure that {

when { UE initiates RA based SDT procedure }

then { UE is successfully able to send and receive subsequent SDT data }

}

(4)

with { UE in NR RRC_INACTIVE state and SDT-CG-Config-r17 is not configured and Random Access resources for RA-SDT are configured }
ensure that {
  when { UE has small data to transmit and the data volume of the pending UL data across all RBs configured for SDT is less than or equal to sdt-DataVolumeThreshold and RSRP is below the configured sdt-RSRP-Threshold }
    then { UE shall not initiate RA based SDT procedure and starts normal RRC Resume procedure }
            }

7.1.1.13.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 38.321, clause 5.1.1b, 5.1.1c, 5.4.4 and 5.27. Unless otherwise stated these are Rel-17 requirements.

[TS 38.321, clause 5.1.1b]

The MAC entity shall:

1>	if the BWP selected for Random Access procedure is configured with both set(s) of Random Access resources with MSG3 repetition indication and set(s) of Random Access resources without MSG3 repetition indication and the RSRP of the downlink pathloss reference is less than rsrp-ThresholdMsg3; or

1>	if the BWP selected for Random Access procedure is only configured with the set(s) of Random Access resources with MSG3 repetition indication:

2>	assume MSG3 repetition is applicable for the current Random Access procedure.

1>	else:

2>	assume MSG3 repetition is not applicable for the current Random Access procedure.

NOTE 1:	Void.

1>	if contention-free Random Access Resources have not been provided for this Random Access procedure and one or more of the features including RedCap and/or a specific NSAG(s) and/or SDT and/or MSG3 repetition is applicable for this Random Access procedure:

NOTE 2: The applicability of SDT is determined by MAC entity according to clause 5.27. The applicability of specific NSAG(s) is determined by upper layers when the Random Access procedure is initiated. The applicability of RedCap is also determined by upper layers when Random Access procedure is initiated and it is applicable to the Random Access procedures initiated by PDCCH orders and any Random Access procedure initiated by the MAC entity.

2>	if none of the sets of Random Access resources are available for any feature applicable to the current Random Access procedure (as specified in clause 5.1.1c):

3>	select the set(s) of Random Access resources that are not associated with any feature indication (as specified in clause 5.1.1c) for this Random Access procedure.

2>	else if there is one set of Random Access resources available which can be used for indicating all features triggering this Random Access procedure:

3>	select this set of Random Access resources for this Random Access procedure.

2>	else (i.e. there are one or more sets of Random Access resources available that are configured with indication(s) for a subset of all features triggering this Random Access procedure):

3>	select a set of Random Access resources from the available set(s) of Random Access resources based on the priority order indicated by upper layers as specified in clause 5.1.1d for this Random Access Procedure.

1>	else if contention-free Random Access Resources have been provided for this Random Access procedure and RedCap is applicable for the current Random Access procedure and there is one set of Random Access resources available that is only configured with RedCap indication:

2>	select this set of Random Access resources for this Random Access procedure.

1>	else:

2>	select the set of Random Access resources that are not associated with any feature indication (as specified in clause 5.1.1c) for the current Random Access procedure.

[TS 38.321, clause 5.1.1c]

The MAC entity shall for each set of configured Random Access resources for 4-step RA type and for each set of configured Random Access resources for 2-step RA type:

1>	if redCap is set to true for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for a Random Access procedure for which RedCap is not applicable.

1>	if smallData is set to true for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for the Random Access procedure which is not triggered for RA-SDT.

1>	if NSAG-List is configured for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for the Random Access procedure unless it is triggered for any one of the NSAG-ID(s) in the NSAG-List.

1>	if msg3-Repetitions is set to true for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for the Random Access procedure if Msg3 repetition is not applicable.

1>	if a set of Random Access resources is not configured with FeatureCombination:

2>	consider the set of Random Access resources to not associated with any feature.

[TS 38.321, clause 5.27]

The MAC entity may be configured by RRC with SDT and the SDT procedure may be initiated by RRC layer. The SDT procedure can be performed either by Random Access procedure with 2-step RA type or 4-step RA type (i.e., RA-SDT) or by configured grant Type 1 (i.e., CG-SDT).

…

If RA-SDT is selected above and after the Random Access procedure is successfully completed (see clause 5.1.6), the UE monitors PDCCH addressed to C-RNTI received in random access response until the RA-SDT procedure is terminated. If CG-SDT is selected above and after the initial transmission for CG-SDT is performed, the UE monitors PDCCH addressed to C-RNTI as stored in UE Inactive AS context as specified in TS 38.331 [5] and CS-RNTI until the CG-SDT procedure is terminated.

7.1.1.13.2.3	Test description

7.1.1.13.2.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

UE:

-	None.

Preamble:

-	The UE is in state 3N-A and Test Mode Activated according to TS 38.508-1 [4] Table 4.4A.2-3 with UE test loop mode B is established IP PDU delay set to 6 seconds. IF pc_logicalChannelSR_DelayTimer the DRB is configured for SDT operation according to Table 7.1.1.13.2.3.1-1.

Table 7.1.1.13.2.3.1-1: Logical Channel Configuration Settings

| Parameter | SDT DRB |
| --- | --- |
| logicalChannelSR-DelayTimerApplied | True |
| logicalChannelSR-DelayTimer | sf512 |

7.1.1.13.2.3.2	Test procedure sequence

Table 7.1.1.13.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits a downlink assignment including the C-RNTI assigned to the UE | <-- | (PDCCH (C-RNTI)) | - | - |
| 2 | SS transmits in the indicated downlink assignment a RLC PDU in a MAC PDU on the DRB configured with SDT (SDT Data > sdt-DataVolumeThreshold). | <-- | MAC PDU | - | - |
| 3 | The SS transmits an RRCRelease message including sdt-Config-r17 in suspendConfig. | <-- | NR RRC: RRCRelease | - | - |
| 4 | Check: Does the UE transmit a preamble on PRACH using a preamble outside of the SmallData FeatureCombinationPreambles? | --> | PRACH Preamble | 2 | P |
| 5 | The SS transmits Random Access Response with RAPID corresponding to the transmitted Preamble in step 4, including TC-RNTI and not including Backoff Indicator subheader. | <-- | Random Access Response | - | - |
| 6 | Check: Does the UE transmit a MAC PDU containing an RRCResumeRequest message? | --> | MAC PDU ( NR RRC: RRCResumeRequest) | 2 | P |
| 7 | The SS schedules PDCCH transmission addressed to TC-RNTI to transmit a valid MAC PDU containing ‘UE Contention Resolution Identity’ MAC control element with matched ‘Contention Resolution Identity’. | <-- | MAC PDU (UE Contention Resolution Identity MAC CE) | - | - |
| 8 | The SS transmits a RRCResume message. | <-- | NR RRC: RRCResume | - | - |
| - | EXCEPTION: Steps 9 and 10 can happen in any order | - | - | - | - |
| 9 | The UE transmits a RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 10 | Check: Does the UE transmit a MAC PDU containing Loop backed PDU? | --> | MAC PDU (containing 1 MAC sub PDU containing RLC SDU) | 2 | P |
| 11 | The SS transmits an OPEN UE TEST LOOP message. | <-- | TC: OPEN UE TEST LOOP | - | - |
| 12 | The UE transmits an OPEN UE TEST LOOP COMPLETE message. | --> | TC: OPEN UE TEST LOOP COMPLETE | - | - |
| 13 | SS transmits a CLOSE UE TEST LOOP message. | <-- | TC: CLOSE UE TEST LOOP | - | - |
| 14 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message. | --> | TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| 15 | SS transmits a downlink assignment including the C-RNTI assigned to the UE | <-- | (PDCCH (C-RNTI)) | - | - |
| 16 | SS transmits in the indicated downlink assignment a RLC PDU in a MAC PDU on the DRB configured with SDT (SDT Data < sdt-DataVolumeThreshold). | <-- | MAC PDU | - | - |
| 17 | The SS transmits a RRCRelease message including sdt-Config-r17 in suspendConfig. | <-- | NR RRC: RRCRelease | - | - |
| 18 | Check: Does the UE transmit a preamble on PRACH using a preamble in the SmallData FeatureCombinationPreambles? | --> | PRACH Preamble | 1 | P |
| 19 | The SS transmits Random Access Response with RAPID corresponding to the transmitted Preamble in step 18, including TC-RNTI and not including Backoff Indicator subheader. | <-- | Random Access Response | - | - |
| 20 | Check: Does the UE transmit a MAC PDU containing an RRCResumeRequest message and RLC PDU on DRB with SDT configured? | --> | MAC PDU ( NR RRC: RRCResumeRequest, RLC PDU on DRB with SDT configured) | 1 | P |
| 21 | The SS schedules PDCCH transmission addressed to TC-RNTI to transmit a valid MAC PDU containing ‘UE Contention Resolution Identity’ MAC control element with matched Contention Resolution Identity’. | <-- | MAC PDU (UE Contention Resolution Identity MAC CE) | - | - |
| - | EXCEPTION: Steps 22a1-22a3 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that takes place if pc_logicalChannelSR_DelayTimer is configured | - | - | - | - |
| 22a1 | IF pc_logicalChannelSR_DelayTimer THEN SS transmits in the indicated downlink assignment a RLC PDU in a MAC PDU on the DRB configured with SDT (SDT Data <= sdt-DataVolumeThreshold). | <-- | MAC PDU |  |  |
| 22a2 | SS transmits an UL Grant, allowing the UE to return the RLC SDU as received in step 22a1, on PDCCH with the C-RNTI assigned to the UE. | <-- | (UL Grant (C-RNTI)) | - | - |
| 22a3 | Check: Does the UE transmit a MAC PDU including one RLC SDU? | --> | MAC PDU | 3 | P |
| 23 | The SS transmits an RRCResume message to bring UE to RRC_CONNECTED State. | <-- | NR RRC: RRCResume | - | - |
| 24 | The UE transmits an RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 25 | The SS changes the parameter ‘sdt-RSRP-Threshold-r17’ in SIB1 of NR Cell 1 to 76 and starts broadcasting updated SIB1. Note: This value should result in meeting condition ‘RSRP is below the configured sdt-RSRP-Threshold’ | - | - | - | - |
| 26 | The SS transmits an OPEN UE TEST LOOP message. | <-- | TC: OPEN UE TEST LOOP | - | - |
| 27 | The UE transmits an OPEN UE TEST LOOP COMPLETE message. | --> | TC: OPEN UE TEST LOOP COMPLETE | - | - |
| 28 | SS transmits a CLOSE UE TEST LOOP message. | <-- | TC: CLOSE UE TEST LOOP | - | - |
| 29 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message. | --> | TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| 30 | SS transmits a downlink assignment including the C-RNTI assigned to the UE | <-- | (PDCCH (C-RNTI)) | - | - |
| 31 | SS transmits in the indicated downlink assignment a RLC PDU in a MAC PDU on the DRB configured with SDT (SDT Data < sdt-DataVolumeThreshold). | <-- | MAC PDU | - | - |
| 32 | The SS transmits a RRCRelease message including sdt-Config-r17 in suspendConfig. | <-- | NR RRC: RRCRelease | - | - |
| 33 | The UE transmits a preamble on PRACH | --> | PRACH Preamble | - | - |
| 34 | The SS transmits Random Access Response with RAPID corresponding to the transmitted Preamble in step 5, including TC-RNTI and not including Backoff Indicator subheader. | <-- | Random Access Response | - | - |
| 35 | Check: Does the UE transmit a MAC PDU containing an RRCResumeRequest message? | --> | MAC PDU ( NR RRC: RRCResumeRequest) | 4 | P |
| 36 | The SS schedules PDCCH transmission addressed to TC-RNTI to transmit a valid MAC PDU containing ‘UE Contention Resolution Identity’ MAC control element with matched ‘Contention Resolution Identity’. | <-- | MAC PDU (UE Contention Resolution Identity MAC CE) | - | - |
| 37 | The SS transmits a RRCResume message. | <-- | NR RRC: RRCResume | - | - |
| - | EXCEPTION: Steps 38 and 39 can happen in any order | - | - | - | - |
| 38 | The UE transmits a RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 39 | Check: Does the UE transmit a MAC PDU containing Loop backed PDU? | --> | MAC PDU (containing 1 MAC sub PDU containing RLC SDU) | 2 | P |
| 40 | The SS transmits a RRCRelease message | <-- | NR RRC: RRCRelease | - | - |

7.1.1.13.2.3.3	Specific message contents

Table 7.1.1.13.2.3.3-1: CLOSE UE TEST LOOP (Steps 13, 28 and preamble Table 7.1.1.13.2.3.2-1)

| Derivation Path: TS 36.508 [7] table 4.7A-3 condition UE test loop mode B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE test loop mode B LB setup |  |  |  |
| IP PDU delay | '0000 0110'B | 6 seconds |  |  |

Table 7.1.1.13.2.3.3-2: SIB1 (preamble and step 25, Table 7.1.1.13.2.3.2-1)

| Derivation path: TS 38.508-1 [4] Table 4.6.1-28 with condition SDT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| servingCellConfigCommon | ServingCellConfigCommon | Table 7.1.1.13.2.3.3-3 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sdt-ConfigCommon-r17 SEQUENCE { |  |  |  |
| sdt-RSRP-Threshold-r17 | 66 (-90dBm) |  | Preamble |
|  | 76 (-80dBm) |  | Step 23 |
| sdt-LogicalChannelSR-DelayTimer-r17 | sf512 |  |  |
| sdt-DataVolumeThreshold-r17 | byte32 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.2.3.3-3: ServingCellConfigCommon (Table 7.1.1.13.2.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| downlinkConfigCommon | DownlinkConfigCommon | Table 7.1.1.13.2.3.3-7 |  |
| uplinkConfigCommon SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkCommon | Table 7.1.1.13.2.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.2.3.3-4: BWP-UplinkCommon (Table 7.1.1.13.2.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkCommon ::= SEQUENCE { |  |  |  |
| AdditionalRACH-ConfigList-r17 SEQUENCE (SIZE(1..maxAdditionalRACH-r17)) OF AdditionalRACH-Config-r17 { | 1 entry |  |  |
| AdditionalRACH-Config-r17[1] SEQUENCE { |  | Entry 1 |  |
| rach-ConfigCommon-r17 | RACH-ConfigCommon | Table 7.1.1.13.2.3.3-5 |  |
| msgA-ConfigCommon-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.2.3.3-5: RACH-ConfigCommon (Table 7.1.1.13.2.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-128 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon::= SEQUENCE { |  |  |  |
| featureCombinationPreamblesList-r17 SEQUENCE (SIZE(1..maxFeatureCombPreamblesPerRACHResource-r17)) OF FeatureCombinationPreambles-r17 { |  |  |  |
| FeatureCombinationPreambles-r17 | FeatureCombinationPreambles | Table 7.1.1.13.2.3.3-6 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.2.3.3-6: FeatureCombinationPreambles (Table 7.1.1.13.2.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-56E |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FeatureCombinationPreambles-r17 ::= SEQUENCE { |  |  |  |
| featureCombination-r17 ::= SEQUENCE { |  |  |  |
| smallData-r17 | true |  |  |
| } |  |  |  |
| startPreambleForThisPartition-r17 | 8 | Randomly selected |  |
| numberOfPreamblesPerSSB-ForThisPartition-r17 | 12 |  |  |
| ssb-SharedRO-MaskIndex-r17 | Not present |  |  |
| groupBconfigured-r17 | Not present |  |  |
| separateMsgA-PUSCH-Config-r17 | Not present |  |  |
| msgA-RSRP-Threshold-r17 | Not present |  |  |
| rsrp-ThresholdSSB-r17 | Not present |  |  |
| deltaPreamble-r17 | Not present |  |  |
| } |  |  |  |

Table 7.1.1.13.2.3.3-7: DownlinkConfigCommon (Table 7.1.1.13.2.3.3-3)

| Derivation Path: TS 38.508-1 [4] table 4.6.3-52 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DownlinkConfigCommon ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkCommon | Table 7.1.1.13.2.3.3-8 |  |
| } |  |  |  |

Table 7.1.1.13.2.3.3-8: BWP-DownlinkCommon (Table 7.1.1.13.2.3.3-7)

| Derivation Path: TS 38.508-1 [4] table 4.6.3-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkCommon ::= SEQUENCE { |  |  |  |
| pdcch-ConfigCommon CHOICE { |  |  |  |
| setup | PDCCH-ConfigCommon with condition SDT |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.2.3.3-9: RRCRelease (Steps 3, 17 and 32 Table 7.1.1.13.2.3.2-1 )

| Derivation Path: TS 38.508-1 [4] table 4.6.1-16 with condition NR_RRC_INACTIVE and SDT |  |  |  |
| --- | --- | --- | --- |
| RRCRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |
| suspendConfig SEQUENCE { |  |  |  |
| sdt-Config-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sdt-DRB-List-r17 SEQUENCE (SIZE (0..maxDRB)) OF DRB-Identity { | 1 entry |  |  |
| DRB-Identity[1] | DRB-Identity using condition DRBj | Entry 1 j is the ID of the DRB established during the preamble which is allocated according to internal TTCN mapping |  |
| } |  |  |  |
| sdt-SRB2-Indication-r17 | Not present |  |  |
| sdt-MAC-PHY-CG-Config-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 7.1.1.13.3	RA Based SDT / 2-step RACH / not complete / RA_TYPE to 4-stepRA

7.1.1.13.3.1	Test Purpose (TP)

(1)

with { UE in NR RRC_INACTIVE state and SDT-CG-Config-r17 is not configured, UE has small data to transmit and initiated 2-step RA based SDT procedure and transmitted MSGA }

ensure that {

when { UE receives the MSGB containing a fallbackRAR MAC subPDU }

then { UE shall fallback to 4-step RA based SDT procedure and initiate msg3 transmission }

}

7.1.1.13.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 38.321, clause 5.1.1b, 5.1.1c and 5.1.4a. Unless otherwise stated these are Rel-17 requirements.

[TS 38.321, clause 5.1.1b]

The MAC entity shall:

1>	if the BWP selected for Random Access procedure is configured with both set(s) of Random Access resources with MSG3 repetition indication and set(s) of Random Access resources without MSG3 repetition indication and the RSRP of the downlink pathloss reference is less than rsrp-ThresholdMsg3; or

1>	if the BWP selected for Random Access procedure is only configured with the set(s) of Random Access resources with MSG3 repetition indication:

2>	assume MSG3 repetition is applicable for the current Random Access procedure.

1>	else:

2>	assume MSG3 repetition is not applicable for the current Random Access procedure.

NOTE 1:	Void.

1>	if contention-free Random Access Resources have not been provided for this Random Access procedure and one or more of the features including RedCap and/or a specific NSAG(s) and/or SDT and/or MSG3 repetition is applicable for this Random Access procedure:

NOTE 2: The applicability of SDT is determined by MAC entity according to clause 5.27. The applicability of specific NSAG(s) is determined by upper layers when the Random Access procedure is initiated. The applicability of RedCap is also determined by upper layers when Random Access procedure is initiated and it is applicable to the Random Access procedures initiated by PDCCH orders and any Random Access procedure initiated by the MAC entity.

2>	if none of the sets of Random Access resources are available for any feature applicable to the current Random Access procedure (as specified in clause 5.1.1c):

3>	select the set(s) of Random Access resources that are not associated with any feature indication (as specified in clause 5.1.1c) for this Random Access procedure.

2>	else if there is one set of Random Access resources available which can be used for indicating all features triggering this Random Access procedure:

3>	select this set of Random Access resources for this Random Access procedure.

2>	else (i.e. there are one or more sets of Random Access resources available that are configured with indication(s) for a subset of all features triggering this Random Access procedure):

3>	select a set of Random Access resources from the available set(s) of Random Access resources based on the priority order indicated by upper layers as specified in clause 5.1.1d for this Random Access Procedure.

1>	else if contention-free Random Access Resources have been provided for this Random Access procedure and RedCap is applicable for the current Random Access procedure and there is one set of Random Access resources available that is only configured with RedCap indication:

2>	select this set of Random Access resources for this Random Access procedure.

1>	else:

2>	select the set of Random Access resources that are not associated with any feature indication (as specified in clause 5.1.1c) for the current Random Access procedure.

[TS 38.321, clause 5.1.1c]

The MAC entity shall for each set of configured Random Access resources for 4-step RA type and for each set of configured Random Access resources for 2-step RA type:

1>	if redCap is set to true for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for a Random Access procedure for which RedCap is not applicable.

1>	if smallData is set to true for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for the Random Access procedure which is not triggered for RA-SDT.

1>	if NSAG-List is configured for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for the Random Access procedure unless it is triggered for any one of the NSAG-ID(s) in the NSAG-List.

1>	if msg3-Repetitions is set to true for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for the Random Access procedure if Msg3 repetition is not applicable.

1>	if a set of Random Access resources is not configured with FeatureCombination:

2>	consider the set of Random Access resources to not associated with any feature.

[TS 38.321, clause 5.1.4a]

1>	start the msgB-ResponseWindow at the PDCCH occasion as specified in TS 38.213 [6], clause 8.2A;

1>	monitor the PDCCH of the SpCell for a Random Access Response identified by MSGB-RNTI while the msgB-ResponseWindow is running;

1>	if C-RNTI MAC CE was included in the MSGA:

2>	monitor the PDCCH of the SpCell for Random Access Response identified by the C-RNTI while the msgB-ResponseWindow is running.

1>	if notification of a reception of a PDCCH transmission of the SpCell is received from lower layers:

2>	if the C-RNTI MAC CE was included in MSGA:

…

2>	if a valid (as specified in TS 38.213 [6]) downlink assignment has been received on the PDCCH for the MSGB-RNTI and the received TB is successfully decoded:

3>	if the MSGB contains a MAC subPDU with Backoff Indicator:

4>	set the PREAMBLE_BACKOFF to value of the BI field of the MAC subPDU using Table 7.2-1, multiplied with SCALING_FACTOR_BI.

3>	else:

4>	set the PREAMBLE_BACKOFF to 0 ms.

3>	if the MSGB contains a fallbackRAR MAC subPDU; and

3>	if the Random Access Preamble identifier in the MAC subPDU matches the transmitted PREAMBLE_INDEX (see clause 5.1.3a):

4>	consider this Random Access Response reception successful;

4>	apply the following actions for the SpCell:

5>	process the received Timing Advance Command (see clause 5.2);

5>	indicate the msgA-PreambleReceivedTargetPower and the amount of power ramping applied to the latest Random Access Preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP);

5>	if the Random Access Preamble was not selected by the MAC entity among the contention-based Random Access Preamble(s):

6>	consider the Random Access procedure successfully completed;

6>	process the received UL grant value and indicate it to the lower layers.

5>	else:

6>	set the TEMPORARY_C-RNTI to the value received in the Random Access Response;

6>	if the Msg3 buffer is empty:

7>	obtain the MAC PDU to transmit from the MSGA buffer and store it in the Msg3 buffer;

6>	process the received UL grant value and indicate it to the lower layers and proceed with Msg3 transmission.

7.1.1.13.3.3	Test description

7.1.1.13.3.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

UE:

-	None.

Preamble:

-	The UE is in state 3N-A and Test Mode Activated according to TS 38.508-1 [4] Table 4.4A.2-3 with UE test loop mode B is established IP PDU delay set to 6 seconds, the DRB is configured for SDT operation.

7.1.1.13.3.3.2	Test procedure sequence

Table 7.1.1.13.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits a downlink assignment including the C-RNTI assigned to the UE. | <-- | (PDCCH (C-RNTI)) | - | - |
| 2 | SS transmits in the indicated downlink assignment a RLC PDU in a MAC PDU on the DRB configured with SDT (SDT Data < sdt-DataVolumeThreshold). | <-- | MAC PDU | - | - |
| 3 | The SS transmits an RRCRelease message including sdt-Config-r17 in suspendConfig. | <-- | NR RRC: RRCRelease | - | - |
| 4 | The UE transmits MSGA using preamble on PRACH after IP PDU Delay expires. | --> | MAC PDU (including NR RRC: RRCResumeRequest, RLC PDU on DRB with SDT configured) | - | - |
| 5 | The SS schedules PDCCH transmission addressed to MSGB-RNTI to transmit a valid MSGB DL MAC PDU containing a fallbackRAR MAC subPDU. | <-- | MAC PDU (fallbackRAR MAC subPDU) | - | - |
| 6 | Check: Does the UE transmit a MAC PDU containing an RRCResumeRequest message and RLC PDU on DRB with SDT configured? | --> | MAC PDU ( NR RRC: RRCResumeRequest, RLC PDU on DRB with SDT configured) | 1 | P |
| 7 | The SS schedules PDCCH transmission addressed to TC-RNTI to transmit a valid MAC PDU containing ‘UE Contention Resolution Identity’ MAC control element with matched ‘Contention Resolution Identity’. | <-- | MAC PDU (UE Contention Resolution Identity MAC CE) | - | - |
| 8 | The SS transmits a RRCResume message. | <-- | NR RRC: RRCResume | - | - |
| 9 | The UE transmits a RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 10 | The SS transmits a RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |

7.1.1.13.3.3.3	Specific message contents

Table 7.1.1.13.3.3.3-0: MAC-CellGroupConfig (preamble)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-68 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MAC-CellGroupConfig ::= SEQUENCE { |  |  |  |
| tag-Config SEQUENCE { |  |  |  |
| tag-ToAddModList SEQUENCE (SIZE (1..maxNrofTAGs)) OF TAG { | 1 entry |  |  |
| TAG[1] SEQUENCE { |  | entry 1 |  |
| timeAlignmentTimer | ms750 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.3.3.3-1: CLOSE UE TEST LOOP (Preamble Table 7.1.1.13.3.3.2-1)

| Derivation Path: TS 36.508-1 [7] table 4.7A-3 condition UE test loop mode B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE test loop mode B LB setup |  |  |  |
| IP PDU delay | '0000 0110'B | 6 seconds | preamble |  |

Table 7.1.1.13.3.3.3-2: FeatureCombinationPreambles (Table 7.1.1.13.3.3.3-6A)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-56E |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FeatureCombinationPreambles-r17 ::= SEQUENCE { |  |  |  |
| featureCombination-r17 ::= SEQUENCE { |  |  |  |
| smallData-r17 | true |  |  |
| } |  |  |  |
| startPreambleForThisPartition-r17 | 8 | Randomly selected |  |
| numberOfPreamblesPerSSB-ForThisPartition-r17 | 12 |  |  |
| ssb-SharedRO-MaskIndex-r17 | Not present |  |  |
| groupBconfigured-r17 | Not present |  |  |
| separateMsgA-PUSCH-Config-r17 | MsgA-PUSCH-Config | TS 38.508-1 [4] table 4.6.3-81B |  |
| msgA-RSRP-Threshold-r17 | 57 | -100 dBm |  |
| rsrp-ThresholdSSB-r17 | RSRP-Range | TS 38.508-1 [4] table 4.6.3-152 |  |
| deltaPreamble-r17 | Not present |  |  |
| } |  |  |  |

Table 7.1.1.13.3.3.3-3: SIB 1 (preamble Table 7.1.1.13.3.3.2-1)

| Derivation path: TS 38.508-1 [4] Table 4.6.1-28 with Condition SDT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| servingCellConfigCommon | ServingCellConfigCommon | Table 7.1.1.13.3.3.3-4 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sdt-ConfigCommon-r17 SEQUENCE { |  |  |  |
| sdt-DataVolumeThreshold-r17 | byte32 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.3.3.3-4: ServingCellConfigCommon (Table 7.1.1.13.3.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| uplinkConfigCommon SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkCommon | Table 7.1.1.13.3.3.3-5 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.3.3.3-5: BWP-UplinkCommon (Table 7.1.1.13.3.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkCommon ::= SEQUENCE { |  |  |  |
| AdditionalRACH-ConfigList-r17 SEQUENCE { |  |  |  |
| AdditionalRACH-Config-r17 SEQUENCE { |  |  |  |
| rach-ConfigCommon -r17 | Not present |  |  |
| msgA-ConfigCommon-r17 | MsgA-ConfigCommon-r16 | Table 7.1.1.13.3.3.3-6 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.3.3.3-6: MsgA-ConfigCommon-r16 (Table 7.1.1.13.3.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-81A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MsgA-ConfigCommon-r16 :: = SEQUENCE { |  |  |  |
| rach-ConfigCommonTwoStepRA-r16 | RACH-ConfigCommonTwoStepRA-r16 | Table 7.1.1.13.3.3.3-6A |  |
| } |  |  |  |

Table 7.1.1.13.3.3.3-6A: RACH-ConfigCommonTwoStepRA-r16 (Table 7.1.1.13.3.3.3-6)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-128A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommonTwoStepRA-r16 ::= SEQUENCE { |  |  |  |
| featureCombinationPreamblesList-r17 SEQUENCE (SIZE(1..maxFeatureCombPreamblesPerRACHResource-r17)) OF FeatureCombinationPreambles-r17 { |  |  |  |
| FeatureCombinationPreambles-r17 | FeatureCombinationPreambles | Table 7.1.1.13.3.3.3-2 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.3.3.3-7: RRCRelease (Step 3 Table 7.1.1.13.3.3.2-1)

| Derivation Path: TS 38.508-1 [4] table 4.6.1-16 with condition NR_RRC_INACTIVE and SDT |  |  |  |
| --- | --- | --- | --- |
| RRCRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |
| suspendConfig SEQUENCE { |  |  |  |
| sdt-Config-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sdt-DRB-List-r17 SEQUENCE (SIZE (0..maxDRB)) OF DRB-Identity { | 1 entry |  |  |
| DRB-Identity[1] | DRB-Identity using condition DRBj | Entry 1 j is the ID of the DRB established during the preamble which is allocated according to internal TTCN mapping |  |
| } |  |  |  |
| sdt-SRB2-Indication-r17 | Not present |  |  |
| sdt-MAC-PHY-CG-Config-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 7.1.1.13.4	RA Based SDT / 4-step RA based SDT / Time Alignment Timer expiry

7.1.1.13.4.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state with TimeAlignmentTimer expired and SDT-CG-Config-r17 is not configured }

ensure that {

when { UE has small data to transmit and the data volume of the pending UL data across all RBs configured for SDT is less than or equal to sdt-DataVolumeThreshold and RSRP is above the configured sdt-RSRP-Threshold }

then { UE shall initiate 4-step RA based SDT procedure }

}

7.1.1.13.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 38.321, clause 5.1.1b, 5.1.1c and 5.2. Unless otherwise stated these are Rel-17 requirements.

[TS 38.321, clause 5.1.1b]

The MAC entity shall:

1>	if the BWP selected for Random Access procedure is configured with both set(s) of Random Access resources with MSG3 repetition indication and set(s) of Random Access resources without MSG3 repetition indication and the RSRP of the downlink pathloss reference is less than rsrp-ThresholdMsg3; or

1>	if the BWP selected for Random Access procedure is only configured with the set(s) of Random Access resources with MSG3 repetition indication:

2>	assume MSG3 repetition is applicable for the current Random Access procedure.

1>	else:

2>	assume MSG3 repetition is not applicable for the current Random Access procedure.

NOTE 1:	Void.

1>	if contention-free Random Access Resources have not been provided for this Random Access procedure and one or more of the features including RedCap and/or a specific NSAG(s) and/or SDT and/or MSG3 repetition is applicable for this Random Access procedure:

NOTE 2: The applicability of SDT is determined by MAC entity according to clause 5.27. The applicability of specific NSAG(s) is determined by upper layers when the Random Access procedure is initiated. The applicability of RedCap is also determined by upper layers when Random Access procedure is initiated and it is applicable to the Random Access procedures initiated by PDCCH orders and any Random Access procedure initiated by the MAC entity.

2>	if none of the sets of Random Access resources are available for any feature applicable to the current Random Access procedure (as specified in clause 5.1.1c):

3>	select the set(s) of Random Access resources that are not associated with any feature indication (as specified in clause 5.1.1c) for this Random Access procedure.

2>	else if there is one set of Random Access resources available which can be used for indicating all features triggering this Random Access procedure:

3>	select this set of Random Access resources for this Random Access procedure.

2>	else (i.e. there are one or more sets of Random Access resources available that are configured with indication(s) for a subset of all features triggering this Random Access procedure):

3>	select a set of Random Access resources from the available set(s) of Random Access resources based on the priority order indicated by upper layers as specified in clause 5.1.1d for this Random Access Procedure.

1>	else if contention-free Random Access Resources have been provided for this Random Access procedure and RedCap is applicable for the current Random Access procedure and there is one set of Random Access resources available that is only configured with RedCap indication:

2>	select this set of Random Access resources for this Random Access procedure.

1>	else:

2>	select the set of Random Access resources that are not associated with any feature indication (as specified in clause 5.1.1c) for the current Random Access procedure.

[TS 38.321, clause 5.1.1c]

The MAC entity shall for each set of configured Random Access resources for 4-step RA type and for each set of configured Random Access resources for 2-step RA type:

1>	if redCap is set to true for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for a Random Access procedure for which RedCap is not applicable.

1>	if smallData is set to true for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for the Random Access procedure which is not triggered for RA-SDT.

1>	if NSAG-List is configured for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for the Random Access procedure unless it is triggered for any one of the NSAG-ID(s) in the NSAG-List.

1>	if msg3-Repetitions is set to true for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for the Random Access procedure if Msg3 repetition is not applicable.

1>	if a set of Random Access resources is not configured with FeatureCombination:

2>	consider the set of Random Access resources to not associated with any feature.

[TS 38.321, clause 5.2]

The MAC entity shall:

1>	when a Timing Advance Command MAC CE is received, and if an NTA (as defined in TS 38.211 [8]) has been maintained with the indicated TAG:

2>	apply the Timing Advance Command for the indicated TAG;

2>	if inactivePosSRS-TimeAlignmentTimer is configured and there is ongoing Positioning SRS Transmission in RRC_INACTIVE as in clause 5.26:

3>	start or restart the inactivePosSRS-TimeAlignmentTimer associated with the indicated TAG.

2>	if CG-SDT procedure triggered as in clause 5.27 is ongoing:

3>	start or restart the cg-SDT-TimeAlignmentTimer associated with the indicated TAG.

2>	else:

3>	start or restart the timeAlignmentTimer associated with the indicated TAG.

…

1>	when a timeAlignmentTimer expires:

2>	if the timeAlignmentTimer is associated with the PTAG:

3>	flush all HARQ buffers for all Serving Cells;

3>	notify RRC to release PUCCH for all Serving Cells, if configured;

3>	notify RRC to release SRS for all Serving Cells, if configured;

3>	clear any configured downlink assignments and configured uplink grants;

3>	clear any PUSCH resource for semi-persistent CSI reporting;

3>	consider all running timeAlignmentTimers as expired;

3>	maintain NTA (defined in TS 38.211 [8]) of all TAGs.

7.1.1.13.4.3	Test description

7.1.1.13.4.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

UE:

-	None.

Preamble:

-	The UE is in state 3N-A and Test Mode Activated according to TS 38.508-1 [4] Table 4.4A.2-3 with UE test loop mode B is established IP PDU delay set to 6 seconds, the DRB is configured for SDT operation.

7.1.1.13.4.3.2	Test procedure sequence

Table 7.1.1.13.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits a downlink assignment including the C-RNTI assigned to the UE | <-- | (PDCCH (C-RNTI)) | - | - |
| 2 | SS transmits in the indicated downlink assignment a RLC PDU in a MAC PDU on the DRB configured with SDT (SDT Data < sdt-DataVolumeThreshold). | <-- | MAC PDU | - | - |
| 3 | The SS transmits an RRCRelease message including sdt-Config-r17 in suspendConfig. | <-- | NR RRC: RRCRelease | - | - |
| 4 | The UE transmits a preamble on PRACH. | --> | PRACH Preamble | - | - |
| 5 | The SS transmits Random Access Response with RAPID corresponding to the transmitted Preamble in step 4, including TC-RNTI and not including Backoff Indicator subheader. TimeAlignmentTimer is started in UE. | <-- | Random Access Response | - | - |
| 6 | The UE transmits a MAC PDU containing an RRCResumeRequest message and RLC PDU on DRB with SDT configured. | --> | MAC PDU ( NR RRC: RRCResumeRequest, RLC PDU on DRB with SDT configured) | - | - |
| 7 | The SS schedules PDCCH transmission addressed to TC-RNTI to transmit a valid MAC PDU containing ‘UE Contention Resolution Identity’ MAC control element with matched ‘Contention Resolution Identity’. | <-- | MAC PDU (UE Contention Resolution Identity MAC CE) | - | - |
| 8 | The SS transmits a RRCResume message. | <-- | NR RRC: RRCResume | - | - |
| 9 | The UE transmits a RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 10 | The SS transmits an OPEN UE TEST LOOP message. | <-- | TC: OPEN UE TEST LOOP | - | - |
| 11 | The UE transmits an OPEN UE TEST LOOP COMPLETE message. | --> | TC: OPEN UE TEST LOOP COMPLETE | - | - |
| 12 | SS transmits a CLOSE UE TEST LOOP message. | <-- | TC: CLOSE UE TEST LOOP | - | - |
| 13 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message. | --> | TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| 14 | SS transmits Timing Advance SS does not send any subsequent timing alignments. TimeAlignmentTimer is re-started in UE. | <-- | MAC PDU (Timing Advance Command MAC Control Element) | - | - |
| 15 | The SS transmits a RLC PDU in a MAC PDU on the DRB configured with SDT (SDT Data < sdt-DataVolumeThreshold). | <-- | MAC PDU | - | - |
| 16 | Check: Does the UE transmit a preamble on PRACH? | --> | PRACH Preamble | 1 | P |
| 17 | The SS transmits Random Access Response with RAPID corresponding to the transmitted Preamble in step 16, including TC-RNTI and not including Backoff Indicator subheader. | <-- | Random Access Response | - | - |
| 18 | Check: Does the UE transmit a MAC PDU containing RLC PDU on DRB with SDT configured? | --> | MAC PDU ( RLC PDU on DRB with SDT configured) | 1 | P |
| 19 | The SS schedules PDCCH transmission addressed to TC-RNTI to transmit a valid MAC PDU containing ‘UE Contention Resolution Identity’ MAC control element with matched ‘Contention Resolution Identity’. | <-- | MAC PDU (UE Contention Resolution Identity MAC CE) | - | - |
| 20 | The SS transmits a RRCRelease message | <-- | NR RRC: RRCRelease | - | - |

7.1.1.13.4.3.3	Specific message contents

Table 7.1.1.13.4.3.3-0: MAC-CellGroupConfig (preamble)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-68 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MAC-CellGroupConfig ::= SEQUENCE { |  |  |  |
| tag-Config SEQUENCE { |  |  |  |
| tag-ToAddModList SEQUENCE (SIZE (1..maxNrofTAGs)) OF TAG { | 1 entry |  |  |
| TAG[1] SEQUENCE { |  | entry 1 |  |
| timeAlignmentTimer | ms750 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.4.3.3-1: CLOSE UE TEST LOOP (Step 12, Preamble Table 7.1.1.13.4.3.2-1)

| Derivation Path: TS 36.508-1 [7] table 4.7A-3 condition UE test loop mode B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE test loop mode B LB setup |  |  |  |
| IP PDU delay | '0000 0110'B | 6 seconds | preamble |  |
| IP PDU delay | '0000 0010'B | 2 seconds > TimeAlignmentTimer | Step 12 |  |

Table 7.1.1.13.4.3.3-2: FeatureCombinationPreambles (Table 7.1.1.13.4.3.3-6)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-56E |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FeatureCombinationPreambles-r17 ::= SEQUENCE { |  |  |  |
| featureCombination-r17 SEQUENCE { |  |  |  |
| smallData-r17 | true |  |  |
| } |  |  |  |
| startPreambleForThisPartition-r17 | 8 | Randomly selected |  |
| numberOfPreamblesPerSSB-ForThisPartition-r17 | 12 |  |  |
| ssb-SharedRO-MaskIndex-r17 | Not present |  |  |
| groupBconfigured-r17 | Not present |  |  |
| separateMsgA-PUSCH-Config-r17 | Not present |  |  |
| msgA-RSRP-Threshold-r17 | Not present |  |  |
| rsrp-ThresholdSSB-r17 | Not present |  |  |
| deltaPreamble-r17 | Not present |  |  |
| } |  |  |  |

Table 7.1.1.13.4.3.3-3: SIB 1 (preamble Table 7.1.1.13.4.3.2-1)

| Derivation path: TS 38.508-1 [4] Table 4.6.1-28 with condition SDT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| servingCellConfigCommon | ServingCellConfigCommon | Table 7.1.1.13.4.3.3-4 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sdt-ConfigCommon-r17 SEQUENCE { |  |  |  |
| sdt-DataVolumeThreshold-r17 | byte32 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.4.3.3-4: ServingCellConfigCommon (Table 7.1.1.13.4.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| uplinkConfigCommon SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkCommon | Table 7.1.1.13.4.3.3-5 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.4.3.3-5: BWP-UplinkCommon (Table 7.1.1.13.4.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkCommon ::= SEQUENCE { |  |  |  |
| AdditionalRACH-ConfigList-r17 SEQUENCE { |  |  |  |
| AdditionalRACH-Config-r17 SEQUENCE { |  |  |  |
| rach-ConfigCommon-r17 | RACH-ConfigCommon | Table 7.1.1.13.4.3.3-6 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.4.3.3-6: RACH-ConfigCommon (Table 7.1.1.13.4.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-128 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon::= SEQUENCE { |  |  |  |
| featureCombinationPreamblesList-r17 SEQUENCE (SIZE(1..maxFeatureCombPreamblesPerRACHResource-r17)) OF FeatureCombinationPreambles-r17 { | 1 entry |  |  |
| FeatureCombinationPreambles-r17[1] | FeatureCombinationPreambles | entry 1Table 7.1.1.13.4.3.3-2 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.4.3.3-7: RRCRelease (Step 3, Table 7.1.1.13.4.3.2-1)

| Derivation Path: TS 38.508-1 [4] table 4.6.1-16 with condition NR_RRC_INACTIVE and SDT |  |  |  |
| --- | --- | --- | --- |
| RRCRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |
| suspendConfig SEQUENCE { |  |  |  |
| sdt-Config-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sdt-DRB-List-r17 SEQUENCE (SIZE (0..maxDRB)) OF DRB-Identity { | 1 entry |  |  |
| DRB-Identity[1] | DRB-Identity using condition DRBj | Entry 1 j is the ID of the DRB established during the preamble which is allocated according to internal TTCN mapping |  |
| } |  |  |  |
| sdt-SRB2-Indication-r17 | Not present |  |  |
| sdt-MAC-PHY-CG-Config-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 7.1.1.13.5	RA Based SDT / CG Based SDT/ cg-SDT-TimeAlignmentTimer

7.1.1.13.5.1	Test Purpose (TP)

(1)

with { UE in NR RRC_INACTIVE state and SDT-CG-Config-r17 is configured and Random Access resources for RA-SDT is configured }

ensure that {

when { UE has small data to transmit and the data volume of the pending UL data across all RBs configured for SDT is less than or equal to sdt-DataVolumeThreshold and RSRP is above the configured sdt-RSRP-Threshold and at least one SSB configured for CG-SDT with SS-RSRP above cg-SDT-RSRP-ThresholdSSB and cg-SDT-TimeAlignmentTimer is running}

then { UE shall initiate CG based SDT procedure }

}

(2)

with { UE in NR RRC_INACTIVE state and SDT-CG-Config-r17 is configured and Random Access resources for RA-SDT is configured }

ensure that {

when { UE has small data to transmit and the data volume of the pending UL data across all RBs configured for SDT is less than or equal to sdt-DataVolumeThreshold and RSRP is above the configured sdt-RSRP-Threshold and at least one SSB configured for CG-SDT with SS-RSRP above cg-SDT-RSRP-ThresholdSSB and cg-SDT-TimeAlignmentTimer expires }

then { UE shall initiate RA based SDT procedure }

}

(3)

with { UE in NR RRC_INACTIVE state and CG-SDT is ongoing}

ensure that {

when { cg-SDT-TimeAlignmentTimer expires before receiving network response for the UL CG-SDT transmission with CCCH message }

then { UE considers ongoing CG-SDT procedure as terminated and performs the actions upon going to RRC_IDLE with release cause 'RRC Resume failure' }

}

(4)

with { UE in NR RRC_INACTIVE state and CG-SDT is ongoing}

ensure that {

when { cg-SDT-TimeAlignmentTimer expires after receiving network response for the UL CG-SDT transmission with CCCH message }

then { UE does not consider ongoing CG-SDT procedure as terminated and UE shall perform uplink transmission using legacy Random Access }

}

(5)

with { UE in NR RRC_INACTIVE state and SDT-CG-Config-r17 is configured and Random Access resources for RA-SDT is configured }

ensure that {

when { UE initiates RA based SDT procedure }

then { UE is successfully able to send and receive subsequent SDT data }

}

7.1.1.13.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.321, clauses 5.2, 5.27.1 and 5.27.2; TS 38.331, clauses 5.3.13.5. Unless otherwise stated these are Rel-17 requirements.

[TS 38.321 clause 5.2]

RRC configures the following parameters for the maintenance of UL time alignment:

…

-	cg-SDT-TimeAlignmentTimer which controls how long the MAC entity considers the uplink transmission for CG-SDT to be uplink time aligned.

The MAC entity shall:

…

1>	when the cg-SDT-TimeAlignmentTimer expires:

2>	clear any configured uplink grants;

2>	if a PDCCH addressed to the MAC entity's C-RNTI after initial transmission for the CG-SDT with CCCH message has not been received:

3>	consider ongoing CG-SDT procedure as terminated;

3>	indicate the expiry of cg-SDT-TimeAlignmentTimer to the upper layer.

2>	flush all HARQ buffers;

2>	maintain NTA (defined in TS 38.211 [8]) of this TAG.

…

The MAC entity shall not perform any uplink transmission on a Serving Cell except the Random Access Preamble and MSGA transmission when the timeAlignmentTimer associated with the TAG to which this Serving Cell belongs is not running, CG-SDT procedure is not ongoing or SRS transmission in RRC_INACTIVE as in clause 5.26 is not on-going. Furthermore, when the timeAlignmentTimer associated with the PTAG is not running, CG-SDT procedure is not ongoing and SRS transmission in RRC_INACTIVE as in clause 5.26 is not ongoing, the MAC entity shall not perform any uplink transmission on any Serving Cell except the Random Access Preamble and MSGA transmission on the SpCell. The MAC entity shall not perform any uplink transmission except the Random Access Preamble and MSGA transmission when the cg-SDT-TimeAlignmentTimer is not running during the ongoing CG-SDT procedure as triggered in clause 5.27. The MAC entity shall not perform any uplink transmission except the Random Access Preamble and MSGA transmission when inactivePosSRS-TimeAlignmentTimer is not running during the procedure for SRS transmission in RRC_INACTIVE as in clause 5.26.

[TS 38.321 clause 5.27.1]

The MAC entity shall, if initiated by the upper layers for SDT procedure:

1>	if the data volume of the pending UL data across all RBs configured for SDT is less than or equal to sdt-DataVolumeThreshold; and

NOTE:	For SDT procedure, the MAC entity also considers the suspended RBs configured with SDT for data volume calculation. It is up to the UE's implementation how the UE calculates the data volume for the suspended RBs. Size of the CCCH message is not considered for data volume calculation

1>	if the RSRP of the downlink pathloss reference is higher than sdt-RSRP-Threshold; or

1>	if sdt-RSRP-Threshold is not configured:

2>	if the Serving Cell is configured with supplementary uplink as specified in TS 38.331 [5]; and

2>	if the RSRP of the downlink pathloss reference is less than rsrp-ThresholdSSB-SUL:

3>	select the SUL carrier.

2>	else:

3>	select the NUL carrier.

2>	if CG-SDT is configured on the selected UL carrier, and TA for CG-SDT is valid according to clause 5.27.2 in the first available CG occasion for initial CG-SDT transmission with CCCH message  according to clause 5.8.2; and

2>	if, for each RB having data available for transmission, configuredGrantType1Allowed, if configured, is configured with value true for the corresponding logical channel; and

2>	if at least one SSB configured for CG-SDT with SS-RSRP above cg-SDT-RSRP-ThresholdSSB is available:

3>	indicate to the upper layers that the conditions for initiating SDT procedure are fulfilled;

3>	perform CG-SDT procedure on the selected UL carrier according to clause 5.8.2.

2>	else if a set of Random Access resources for performing RA-SDT are selected according to clause 5.1.1b on the selected UL carrier:

3>	if cg-SDT-TimeAlignmentTimer is running, consider cg-SDT-TimeAlignmentTimer as expired and perform the corresponding actions in clause 5.2;

3>	indicate to the upper layers that the conditions for initiating SDT procedure are fulfilled.

[TS 38.321 clause 5.27.2]

The MAC entity shall consider the TA of the initial CG-SDT transmission with CCCH message to be valid when the following conditions are fulfilled:

1>	The RSRP values for the stored downlink pathloss reference and the current downlink pathloss reference are valid according to TS 38.133 [11]; and

1>	Compared to the stored downlink pathloss reference RSRP value, the current RSRP value of the downlink pathloss reference calculated as specified in TS 38.133 [11] has not increased/decreased by more than cg-SDT-RSRP-ChangeThreshold, if configured; and

1>	cg-SDT-TimeAlignmentTimer is running.

[TS 38.331 clause 5.3.13.5]

The UE shall:

…

1>	else if indication from the MCG RLC that the maximum number of retransmissions has been reached is received while SDT procedure is ongoing; or

1>	if random access problem indication is received from MCG MAC while SDT procedure is ongoing; or

1>	if the lower layers indicate that cg-SDT-TimeAlignmentTimer or the configuredGrantTimer expired before receiving network response for the UL CG-SDT transmission with CCCH message while SDT procedure is ongoing; or

1>	if T319a expires:

2>	consider SDT procedure is not ongoing;

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause 'RRC Resume failure'.

7.1.1.13.5.3	Test description

7.1.1.13.5.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used in NR cell.

UE:

None.

Preamble:

-	The UE is in 5GS state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3 and Test Loop Function (On) with UE test loop mode B is established, if pc_logicalChannelSR_DelayTimer=True the DRB is configured according to Table 7.1.1.13.5.3.1-1 for SDT operation.

Table 7.1.1.13.5.3.1-1: Logical Channel Configuration Settings

| Parameter | SDT DRB |
| --- | --- |
| logicalChannelSR-DelayTimerApplied | True |
| logicalChannelSR-DelayTimer | sf512 |

7.1.1.13.5.3.2	Test procedure sequence

Table 7.1.1.13.5.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a downlink assignment including the C-RNTI assigned to the UE | <-- | (PDCCH (C-RNTI)) | - | - |
| 2 | SS transmits in the indicated downlink assignment a RLC PDU in a MAC PDU on the DRB configured with SDT. (Note 1) | <-- | MAC PDU | - | - |
| 3 | The SS transmits an RRCRelease message including SDT-CG-Config-r17 in suspendConfig. The cg-SDT-TimeAlignmentTimer is configured to 5120ms. | <-- | NR RRC: RRCRelease | - | - |
| 4 | The SS is configured on NR Cell 1 not to send RLC acknowledgement (RLC ACK) to the next received RLC SDU to the UE (Note 2) | - | - | - | - |
| 5 | Check: The UE transmits a MAC PDU containing an RRCResumeRequest message and RLC PDU on DRB with SDT configured in a CG PUSCH occasion? | --> | MAC PDU ( NR RRC: RRCResumeRequest, RLC PDU on DRB with SDT configured) | 1 | P |
| 6 | The SS waits 6s from step 3 to ensure that cg-SDT-TimeAlignmentTimer expired and then transmits Paging message including ng-5G-S-TMSI. (Note 3) | <-- | NR RRC: Paging | - | - |
| 7 | Check: Does the UE transmit an RRCSetupRequest message? | --> | NR RRC: RRCSetupRequest | 3 | P |
| 8-13 | Steps 3 to 8 of the NR RRC_CONNECTED procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 14 | The SS transmits an OPEN UE TEST LOOP message. | <-- | NR RRC: DLInformationTransfer TC: OPEN UE TEST LOOP | - | - |
| 15 | The UE transmits an OPEN UE TEST LOOP COMPLETE message. | --> | NR RRC: ULInformationTransfer TC: OPEN UE TEST LOOP COMPLETE | - | - |
| 16 | The SS transmits a CLOSE UE TEST LOOP message. | <-- | NR RRC: DLInformationTransfer TC: CLOSE UE TEST LOOP | - | - |
| 17 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message. | --> | NR RRC: ULInformationTransfer TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| 18 | The SS transmits a downlink assignment including the C-RNTI assigned to the UE | <-- | (PDCCH (C-RNTI)) | - | - |
| 19 | SS transmits in the indicated downlink assignment a RLC PDU in a MAC PDU on the DRB configured with SDT. (Note 1) | <-- | MAC PDU | - | - |
| 20 | The SS transmits an RRCRelease message including SDT-CG-Config-r17 in suspendConfig. The cg-SDT-TimeAlignmentTimer is configured to 5120ms. | <-- | NR RRC: RRCRelease | - | - |
| 21 | The SS is configured on NR Cell 1 not to send RLC acknowledgement (RLC ACK) to the next received RLC SDU to the UE. (Note 2) | - | - | - | - |
| 22 | Check: The UE transmits a MAC PDU containing an RRCResumeRequest message and RLC PDU on DRB with SDT configured in a CG PUSCH occasion? | --> | MAC PDU ( NR RRC: RRCResumeRequest, RLC PDU on DRB with SDT configured) | 1 | P |
| 23 | The SS waits 6s from step 20 to ensure that cg-SDT-TimeAlignmentTimer expired and then transmits Paging message including matched fullI-RNTI. (Note 3) | <-- | NR RRC: Paging | - | - |
| 24 | Check: Does the UE transmit an RRCResumRequest message within 10s? | --> | NR RRC: RRCResumeRequest | 3 | F |
| 25-32 | The test steps 1 to 8 of generic test procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are performed on NR Cell 1. | - | - | - | - |
| 33 | The SS transmits an OPEN UE TEST LOOP message. | <-- | NR RRC: DLInformationTransfer TC: OPEN UE TEST LOOP | - | - |
| 34 | The UE transmits an OPEN UE TEST LOOP COMPLETE message. | --> | NR RRC: ULInformationTransfer TC: OPEN UE TEST LOOP COMPLETE | - | - |
| 35 | The SS transmits a CLOSE UE TEST LOOP message. | <-- | NR RRC: DLInformationTransfer TC: CLOSE UE TEST LOOP | - | - |
| 36 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message. | --> | NR RRC: ULInformationTransfer TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| 37 | The SS transmits a downlink assignment including the C-RNTI assigned to the UE | <-- | (PDCCH (C-RNTI)) | - | - |
| 38 | SS transmits in the indicated downlink assignment a RLC PDU in a MAC PDU on the DRB configured with SDT. (Note 1) | <-- | MAC PDU | - | - |
| 39 | The SS transmits an RRCRelease message including SDT-CG-Config-r17 in suspendConfig. The cg-SDT-TimeAlignmentTimer is configured to 5120ms. | <-- | NR RRC: RRCRelease | - | - |
| 40 | Check: The UE transmits a MAC PDU containing an RRCResumeRequest message and RLC PDU on DRB in a CG PUSCH occasion? | --> | MAC PDU ( NR RRC: RRCResumeRequest, RLC PDU on DRB) | 1 | P |
| 41 | The SS waits 6s from step 39 to ensure that cg-SDT-TimeAlignmentTimer expired and then transmits a RLC PDU in a MAC PDU on the DRB configured with SDT. | <-- | MAC PDU | - | - |
| 42 | Check: Does the UE transmit a preamble on PRACH with ra-PreambleIndex range from 0 to 7 for FR1 and from 0 to 3 for FR2? | --> | PRACH Preamble | 4 | P |
| 43 | The SS transmits Random Access Response with RAPID corresponding to the transmitted Preamble in step 42, including TC-RNTI and not including Back off Indicator subheader. | <-- | Random Access Response | - | - |
| 44 | Check: Does the UE transmit an RRCResumeRequest message? | --> | MAC PDU ( NR RRC: RRCResumeRequest) | 4 | P |
| 45 | The SS schedules PDCCH transmission addressed to TC-RNTI to transmit a valid MAC PDU containing ‘UE Contention Resolution Identity’ MAC control element with matched ‘Contention Resolution Identity’. | <-- | MAC PDU (UE Contention Resolution Identity MAC CE and NR RRC: RRCResume) | - | - |
| 46 | Void |  |  |  |  |
| - | Exception: Step 47 and 48 can happen in any order | - | - | - | - |
| 47 | The UE transmits an RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 48 | Check: Does the UE transmits a MAC PDU containing Loop backed PDU? | --> | MAC PDU (containing 1 MAC sub PDU containing RLC SDU) | 4 | P |
| - | EXCEPTION: Steps 49a1-49a11 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that takes place if pc_ra_SDT_r17 is true (UE supporting RA-SDT) | - | - | - | - |
| 49a1 | IF pc_ra_SDT_r17 THEN the SS transmits an OPEN UE TEST LOOP message. | <-- | NR RRC: DLInformationTransfer TC: OPEN UE TEST LOOP | - | - |
| 49a2 | The UE transmits an OPEN UE TEST LOOP COMPLETE message. | --> | NR RRC: ULInformationTransfer TC: OPEN UE TEST LOOP COMPLETE | - | - |
| 49a3 | The SS transmits a CLOSE UE TEST LOOP message. | <-- | NR RRC: DLInformationTransfer TC: CLOSE UE TEST LOOP | - | - |
| 49a4 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message. | --> | NR RRC: ULInformationTransfer TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| 49a5 | The SS transmits a downlink assignment including the C-RNTI assigned to the UE | <-- | (PDCCH (C-RNTI)) | - | - |
| 49a6 | The SS transmits in the indicated downlink assignment a RLC PDU in a MAC PDU on the DRB configured with SDT( SDT Data < sdt-DataVolumeThreshold).(Note 1) | <-- | MAC PDU | - | - |
| 49a7 | The SS transmits an RRCRelease message including SDT-CG-Config-r17 in suspendConfig. The cg-SDT-TimeAlignmentTimer is configured to 5120ms. | <-- | NR RRC: RRCRelease | - | - |
| 49a8 | Check: Does the UE transmit a preamble on PRACH with ra-PreambleIndex range from 8 to 15? | --> | PRACH Preamble | 2 | P |
| 49a9 | The SS transmits Random Access Response with RAPID corresponding to the transmitted Preamble in step 49a8, including TC-RNTI and not including Back off Indicator subheader. | <-- | Random Access Response | - | - |
| 49a10 | Check: The UE transmits a MAC PDU containing an RRCResumeRequest message and RLC PDU on DRB with SDT configured? | --> | MAC PDU ( NR RRC: RRCResumeRequest, RLC PDU on DRB with SDT configured) | 2 | P |
| 49a11 | The SS schedules PDCCH transmission addressed to TC-RNTI to transmit a valid MAC PDU containing ‘UE Contention Resolution Identity’ MAC control element with matched ‘Contention Resolution Identity’. | <-- | MAC PDU (UE Contention Resolution Identity MAC CE) | - | - |
| - | EXCEPTION: Steps 49a12a1-49a12a3 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that takes place if pc_logicalChannelSR_DelayTimer is true | - | - | - | - |
| 49a12a1 | IF pc_logicalChannelSR_DelayTimer THEN SS transmits in the indicated downlink assignment a RLC PDU in a MAC PDU on the DRB configured with SDT (SDT Data <= sdt-DataVolumeThreshold) (Note 1) | <-- | MAC PDU | - | - |
| 49a12a2 | SS transmits an UL Grant, allowing the UE to return the RLC SDU as received in step 49a12a1, on PDCCH with the C-RNTI assigned to the UE. | <-- | (UL Grant (C-RNTI)) | - | - |
| 49a12a3 | Check: Does the UE transmit a MAC PDU including one RLC SDU? | --> | MAC PDU | 5 | P |
| 50 | The SS transmits a RRCRelease message | <-- | NR RRC: RRCRelease | - | - |
| Note 1: RLC PDU is 97 bytes (RLC SDU is 94 bytes for 3 bytes RLC header and 95 bytes for 2 bytes RLC header) and sdt-DataVolumeThreshold is 100 bytes. Therefore the size of RLC SDU is less than the sdt-DataVolumeThreshold. Note 2: This step is used to ensure UE could not receive PDCCH addressed to the MAC entity's C-RNTI after initial transmission for the CG-SDT with CCCH message. Note 3: After cg-SDT-TimeAlignmentTimer expired, UE considers ongoing CG-SDT procedure as terminated and performs the actions upon going to RRC_IDLE. Therefore UE could receive CN Paging using 5G-S-TMSI and could not receive RAN Paging using fullI-RNTI. |  |  |  |  |  |

7.1.1.13.5.3.3	Specific message contents

Table 7.1.1.13.5.3.3-1: CLOSE UE TEST LOOP (Preamble and steps 16,35 and 49a3, Table 7.1.1.13.5.3.2-1)

| Derivation path: 36.508-1 [7] table 4.7A-3 condition UE test loop mode B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE test loop mode B LB setup |  |  |  |
| IP PDU delay | '0000 0100'B | 4 seconds | Preamble, Step16, Step35 |  |
|  | '0000 1000'B | 8 seconds | Step49a3 |  |

Table 7.1.1.13.5.3.3-2: SIB1 (Preamble and all steps, Table 7.1.1.13.5.3.2-1)

| Derivation Path: TS 38.508-1 [4] table 4.6.1-28 with condition SDT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| servingCellConfigCommon | ServingCellConfigCommonSIB | Table 7.1.1.13.5.3.3-3 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sdt-ConfigCommon-r17 SEQUENCE { |  |  |  |
| sdt-RSRP-Threshold-r17 | 60 | (IE value – 156) dBm = - 96 dBm |  |
| sdt-LogicalChannelSR-DelayTimer-r17 | sf512 |  |  |
| sdt-DataVolumeThreshold-r17 | byte100 |  |  |
| t319a-r17 | ms4000 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.5.3.3-3: ServingCellConfigCommonSIB (Table 7.1.1.13.5.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| uplinkConfigCommon | UplinkConfigCommonSIB | Table 7.1.1.13.5.3.3-4 |  |
| } |  |  |  |

Table 7.1.1.13.5.3.3-4: UplinkConfigCommonSIB (Table 7.1.1.13.5.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-202 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkConfigCommonSIB ::= SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkCommon | Table 7.1.1.13.5.3.3-5 |  |
| } |  |  |  |

Table 7.1.1.13.5.3.3-5: BWP-UplinkCommon (Table 7.1.1.13.5.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkCommon ::= SEQUENCE { |  |  |  |
| rach-ConfigCommon CHOICE { |  |  |  |
| setup | RACH-ConfigCommon | Table 7.1.1.13.5.3.3-6 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.5.3.3-6: RACH-ConfigCommon (Table 7.1.1.13.5.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-128 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon ::= SEQUENCE { |  |  |  |
| featureCombinationPreamblesList-r17 SEQUENCE (SIZE(1..maxFeatureCombPreamblesPerRACHResource-r17)) OF FeatureCombinationPreambles-r17 { | 1 entry |  |  |
| FeatureCombinationPreambles-r17[1] SEQUENCE { |  | entry 1 |  |
| featureCombination-r17 SEQUENCE { |  |  |  |
| smallData-r17 | True |  |  |
| } |  |  |  |
| startPreambleForThisPartition-r17 | 8 |  |  |
| numberOfPreamblesPerSSB-ForThisPartition-r17 | 8 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.5.3.3-7: RRCRelease (steps 3, 20, 39 and 49a7, Table 7.1.1.13.5.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-16 with condition NR_RRC_INACTIVE and SDT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCRelease ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |
| suspendConfig SEQUENCE { |  |  |  |
| sdt-Config-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sdt-DRB-List-r17 SEQUENCE (SIZE (0..maxDRB)) OF DRB-Identity { | 1 entry |  |  |
| DRB-Identity[1] | DRB-Identity using condition DRBj | Entry 1 j is the ID of the DRB established during the preamble which is allocated according to internal TTCN mapping |  |
| } |  |  |  |
| sdt-MAC-PHY-CG-Config-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cg-SDT-ConfigInitialBWP-NUL-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| pusch-Config-r17 CHOICE { |  |  |  |
| setup | PUSCH-Config | TS 38.508-1[4] Table 4.6.3-118: PUSCH-Config |  |
| } |  |  |  |
| configuredGrantConfigToAddModList-r17 SEQUENCE (SIZE (1..maxNrofConfiguredGrantConfig-r16)) OF ConfiguredGrantConfig { |  |  |  |
| ConfiguredGrantConfig[1] | ConfiguredGrantConfig | Table 7.1.1.13.5.3.3-8 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cg-SDT-ConfigInitialBWP-SUL-r17 | Not present |  |  |
| cg-SDT-ConfigInitialBWP-DL-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| pdcch-Config-r17 | Not present |  |  |
| pdsch-Config-r17 CHOICE { |  |  |  |
| setup | PDSCH-Config | TS 38.508-1[4] Table 4.6.3-100: PDSCH-Config |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cg-SDT-TimeAlignmentTimer-r17 | ms5120 | TimeAlignmentTimer(5.12s) |  |
| cg-SDT-RSRP-ThresholdSSB-r17 | 60 | (IE value – 156) dBm = - 96 dBm |  |
| cg-SDT-TA-ValidationConfig-r17 | Not present | UE does not perform RSRP based TA validation |  |
| } |  |  |  |
| cg-SDT-CS-RNTI-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.5.3.3-8: ConfiguredGrantConfig (Table 7.1.1.13.5.3.3-7)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-26, condition CG_Config_Type1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ConfiguredGrantConfig ::= SEQUENCE { |  |  |  |
| rrc-ConfiguredUplinkGrant SEQUENCE{ |  |  |  |
| frequencyDomainAllocation | BIT STRING (SIZE(18) | Equals to NBWPsize * (NBWPsize – LRB + 1) + (NBWPsize -1 - RBstart), where LRB = 24 PRB, RBstart = 0, NBWPsize is the size [PRBs] of the carrier bandwidth and contained in TS.38.508-1 [4] clause 6.2.3.1 | SCS15 AND CBW5, SCS30 AND CBW10 |
|  | BIT STRING (SIZE(18) | Equal to NBWPsize * (LRB-1) + RBstart), where LRB = 24 PRB, RBstart = 0, NBWPsize is the size [PRBs] of the carrier bandwidth and contained in TS.38.508-1 [4] clause 6.2.3.1 |  |
| dmrs-SeqInitialization | 0 |  |  |
| mcsAndTBS | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| Note : Configured UL grant for CG-SDT is 888 bits (LRBs & IMCS as per 38.523-3[3] annex B) is chosen to enable UE to transmit RRCResumeRequest and Loop backed RLC PDU in a MAC PDU ( 97 bytes for Loop backed RLC PDU + 2 bytes for MAC Sub Header + 6 bytes for RRCResumeRequest message + 2 bytes for MAC Sub Header + 4 bytes for BSR and Padding ) |  |  |  |

Table 7.1.1.13.5.3.3-9: RRCResumeRequest (steps 5, 22, 40 and 49a10, Table 7.1.1.13.5.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCResumeRequest ::= SEQUENCE { |  |  |  |
| rrcResumeRequest SEQUENCE { |  |  |  |
| resumeCause | mo-data |  |  |
| } |  |  |  |
| } |  |  |  |

##### 7.1.1.13.6	RA Based MT-SDT / 2-step RACH/ Success

7.1.1.13.6.1	Test Purpose (TP)

(1)

with { UE in NR RRC_INACTIVE state and SDT-CG-Config is not configured and Random Access resources for 2-step RA-SDT are configured }

ensure that {

when { UE receives paging message with mt-SDT indication included and RSRP is above the configured mt-SDT-RSRP-Threshold }

then { UE shall initiate 2-step RA based mt-SDT procedure using legacy Random Access resources with resumeCause set to mt-SDT in RRC connection resumption procedure }

}

(2)

with { UE in NR RRC_INACTIVE state and SDT-CG-Config is not configured and Random Access resources for 2-step RA-SDT are configured }

ensure that {

when { UE initiates 2-step RA based mt-SDT procedure }

then { UE is successfully able to receive subsequent SDT data }

}

(3)

with { UE in NR RRC_INACTIVE state and SDT-CG-Config is not configured and Random Access resources for 2-step RA-SDT are configured }
ensure that {
  when { UE receives paging message with mt-SDT indication included and RSRP is below the configured mt-SDT-RSRP-Threshold }
    then { UE shall not initiate RA based mt-SDT procedure and starts normal RRC Resume procedure resumeCause set to mt-Access in RRC connection resumption procedure }
            }

7.1.1.13.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 38.321, clause 5.1.1b, 5.1.1c & 5.27, 3GPP TS 38.331, clause 5.3.2.3 & 5.3.13.1b. Unless otherwise stated these are Rel-18 requirements.

[TS 38.321, clause 5.1.1b]

The MAC entity shall:

1>	if the BWP selected for Random Access procedure is configured with both set(s) of Random Access resources with msg3-Repetitions set to true and set(s) of Random Access resources without msg3-Repetitions set to true and the RSRP of the downlink pathloss reference is less than rsrp-ThresholdMsg3; or

1>	if the BWP selected for Random Access procedure is only configured with the set(s) of Random Access resources with msg3-Repetitions set to true:

2>	assume Msg3 repetition is applicable for the current Random Access procedure.

1>	else:

2>	assume Msg3 repetition is not applicable for the current Random Access procedure.

...

1>	if neither contention-free Random Access Resources nor Random Access Resources for SI request have been provided for this Random Access procedure and one or more of the features including (e)RedCap and/or Slicing and/or SDT and/or MSG3 repetition and/or MSG1 repetition is applicable for this Random Access procedure:

NOTE 2:	The applicability of SDT is determined by MAC entity according to clause 5.27. The applicability of NSAG-ID is determined by upper layers when the Random Access procedure is initiated. The applicability of (e)RedCap is also determined by upper layers when Random Access procedure is initiated and it is applicable to the Random Access procedures initiated by PDCCH orders and any Random Access procedure initiated by the MAC entity.

NOTE 3:	SDT is not applicable for the Random Access procedure initiated by upper layers for MT-SDT.

2>	if none of the sets of Random Access resources are available for any feature applicable to the current Random Access procedure (as specified in clause 5.1.1c):

3>	select the set(s) of Random Access resources that are not associated with any feature indication (as specified in clause 5.1.1c) for this Random Access procedure.

2>	else if there is one set of Random Access resources available which can be used for indicating all features triggering this Random Access procedure:

3>	select this set of Random Access resources for this Random Access procedure.

2>	else if there are more than one set of Random Access resources available which can be used for indicating all features triggering this Random Access procedure and Msg1 repetition is applicable for this Random Access procedure:

3>	select the set of Random Access resources that associated with highest repetition number among the sets of Random Access resources.

2>	else (i.e. there are one or more sets of Random Access resources available that are configured with indication(s) for a subset of all features triggering this Random Access procedure):

3>	select a set of Random Access resources from the available set(s) of Random Access resources based on the priority order indicated by upper layers as specified in clause 5.1.1d for this Random Access Procedure.

...

1>	else:

2>	select the set of Random Access resources that is not associated with any feature indication (as specified in clause 5.1.1c) for the current Random Access procedure.

[TS 38.321, clause 5.1.1c]

The MAC entity shall for each set of configured Random Access resources:

1>	if eRedCap is set to true for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for a Random Access procedure for which eRedCap is not applicable.

1>	if redCap is set to true for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for a Random Access procedure for which RedCap is not applicable.

1>	if smallData is set to true for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for the Random Access procedure which is not triggered for RA-SDT by MO-SDT as specified in TS 38.331 [5].

1>	if NSAG-List is configured for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for the Random Access procedure unless it is triggered for any one of the NSAG-ID(s) in the NSAG-List.

1>	if msg3-Repetitions is set to true for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for the Random Access procedure if Msg3 repetition is not applicable.

1>	if msg1-Repetitions is set to true for a set of Random Access resources:

2>	if Msg1 repetition is not applicable to the current Random Access procedure; or

2>	if the set of Random Access resources is not associated with any of the Msg1 repetition number that is applicable to the current Random Access procedure:

3>	consider the set of Random Access resources as not available for the Random Access procedure.

1>	if a set of Random Access resources is not configured with FeatureCombination:

2>	consider the set of Random Access resources to not associated with any feature.

[TS 38.321, clause 5.27]

The MAC entity may be configured by RRC with SDT and the SDT procedure may be initiated by RRC layer for MO-SDT or MT-SDT. The SDT procedure initiated for MO-SDT can be performed either by Random Access procedure with 2-step RA type or 4-step RA type (i.e., RA-SDT) or by configured grant Type 1 (i.e., CG-SDT). The SDT procedure initiated for MT-SDT cannot be performed by RA-SDT (i.e., RA-SDT is not applicable as specified in clause 5.1.1b), but can be performed either by Random Access procedure (i.e., with 2-step RA type or 4-step RA type) or by configured grant Type 1 (i.e., CG-SDT).

RRC configures the following parameters for SDT procedure:

-	sdt-DataVolumeThreshold: data volume threshold for the UE to determine whether to perform SDT procedure initiated for MO-SDT;

-	sdt-RSRP-Threshold: RSRP threshold for UE to determine whether to perform SDT procedure initiated for MO-SDT;

-	mt-SDT-RSRP-Threshold: RSRP threshold for UE to determine whether to perform SDT procedure initiated for MT-SDT;

-	cg-SDT-RSRP-ThresholdSSB: an RSRP threshold configured for SSB selection for CG-SDT;

-	cg-MT-SDT-MaxDurationToNextCG-Occasion: time threshold which is used by the UE to determine whether to perform CG-SDT for MT-SDT;

-	cg-SDT-MaxDurationToNextCG-Occasion: time threshold configured per logical channel which is used by the UE to determine whether to perform CG-SDT for MO-SDT;

-	sdt-BeamFailureRecoveryProhibitTimer: prohibit timer to avoid frequent triggering of Random Access procedure due to beam failure recovery during RA-SDT procedure or during MT-SDT procedure initiated by Random Access procedure.

The following UE variable is used for the SDT procedure:

-	MAX_DURATION_TO_NEXT_CG_OCCASION;

-	RSRP_THRESHOLD.

The MAC entity shall, if initiated by the upper layers for SDT procedure:

1>	if SDT procedure is initiated for MO-SDT as specified in TS 38.331 [5]:

2>	set the MAX_DURATION_TO_NEXT_CG_OCCASION to the shortest value of cg-SDT-MaxDurationToNextCG-Occasion, if configured, among all the logical channels configured with this parameter by upper layer and having data for transmission;

2>	set the RSRP_THRESHOLD to the value of sdt-RSRP-Threshold, if configured.

1>	else if SDT procedure is initiated for MT-SDT as specified in TS 38.331 [5]:

2>	set the MAX_DURATION_TO_NEXT_CG_OCCASION to the value of cg-MT-SDT-MaxDurationToNextCG-Occasion, if configured;

2>	if mt-SDT-RSRP-Threshold is configured:

3>	set the RSRP_THRESHOLD to the value of mt-SDT-RSRP-Threshold.

2>	else if sdt-RSRP-Threshold is configured:

3>	set the RSRP_THRESHOLD to the value of sdt-RSRP-Threshold.

1>	if the SDT procedure is initiated for MO-SDT as specified in TS 38.331 [5], and the data volume of the pending UL data across all RBs configured for SDT is less than or equal to sdt-DataVolumeThreshold, or if the SDT procedure is initiated for MT-SDT as specified in TS 38.331 [5]; and

NOTE 1:	For SDT procedure, the MAC entity also considers the suspended RBs configured with SDT for data volume calculation. It is up to the UE's implementation how the UE calculates the data volume for the suspended RBs. Size of the CCCH message is not considered for data volume calculation

1>	if the RSRP of the downlink pathloss reference is higher than RSRP_THRESHOLD or if RSRP_THRESHOLD is not set:

2>	if the Serving Cell is configured with supplementary uplink as specified in TS 38.331 [5]; and

2>	if the RSRP of the downlink pathloss reference is less than rsrp-ThresholdSSB-SUL:

3>	select the SUL carrier.

2>	else:

3>	select the NUL carrier.

2>	if CG-SDT is configured on the selected UL carrier, and TA for CG-SDT is valid according to clause 5.27.2 in the first available CG occasion for initial CG-SDT transmission with CCCH message according to clause 5.8.2; and

2>	if the SDT procedure is initiated for MO-SDT as specified in TS 38.331 [5], and, for each RB having data available for transmission, configuredGrantType1Allowed, if configured for CG-SDT, is configured with value true for the corresponding logical channel, or if the SDT procedure is initiated for MT-SDT as specified in TS 38.331 [5]; and

2>	if at least one SSB configured for CG-SDT with SS-RSRP above cg-SDT-RSRP-ThresholdSSB is available, and if either the time gap between the initiation of the SDT procedure and first available CG occasion for initial CG-SDT transmission with CCCH message according to clause 5.8.2 is less than MAX_DURATION_TO_NEXT_CG_OCCASION, or if the MAX_DURATION_TO_NEXT_CG_OCCASION is not set:

3>	indicate to the upper layers that the conditions for initiating SDT procedure are fulfilled;

3>	perform CG-SDT procedure on the selected UL carrier according to clause 5.8.2.

2>	else if a set of Random Access resources for RA-SDT is configured and can be selected according to clause 5.1.1b on the selected UL carrier on the BWP configured by initialUplinkBWP-RedCap, if configured for an (e)RedCap UE; otherwise, on the BWP configured by initialUplinkBWP; or

2>	if the SDT procedure is initiated for MT-SDT as specified in TS 38.331 [5]:

3>	if cg-SDT-TimeAlignmentTimer is running, consider cg-SDT-TimeAlignmentTimer as expired and perform the corresponding actions in clause 5.2;

3>	indicate to the upper layers that the conditions for initiating SDT procedure are fulfilled.

2>	else:

3>	indicate to the upper layers that the conditions for initiating SDT procedure are not fulfilled.

1>	else:

2>	indicate to the upper layers that the conditions for initiating SDT procedure are not fulfilled.

If Random Access procedure is selected above for SDT procedure initiated for MO-SDT or MT-SDT and after the Random Access procedure is successfully completed (see clause 5.1.6), the UE monitors PDCCH addressed to C-RNTI received in random access response until the SDT procedure is terminated. If CG-SDT is selected above and after the initial transmission for CG-SDT is performed, the UE monitors PDCCH addressed to C-RNTI as stored in UE Inactive AS context as specified in TS 38.331 [5] and CS-RNTI until the SDT procedure is terminated.

The MAC entity shall:

1>	if sdt-BeamFailureRecoveryProhibitTimer is configured and not running; and

1>	if RA-SDT procedure for MO-SDT or MT-SDT procedure initiated by Random Access procedure is ongoing:

2>	if SS-RSRP of the SSB selected in the last successfully completed Random Access procedure during ongoing RA-SDT procedure or ongoing MT-SDT procedure initiated by Random Access procedure is less than rsrp-ThresholdSSB:

3>	start the sdt-BeamFailureRecoveryProhibitTimer;

3>	initiate a Random Access procedure (see clause 5.1).

NOTE 1a:	It is up to UE implementation when to measure SSBs during ongoing RA-SDT procedure or ongoing MT-SDT procedure initiated by Random Access procedure. Based on implementation the UE need not initiate the Random Access procedure if there is no SSB with SS-RSRP above rsrp-ThresholdSSB. The UE uses rsrp-ThresholdSSB in Random Access configuration selected by the UE when RA-SDT or MT-SDT procedure was initiated.

NOTE 2:	When the UE determines if there is an SSB with SS-RSRP above cg-SDT-RSRP-ThresholdSSB or less than rsrp-ThresholdSSB, the UE uses the latest unfiltered L1-RSRP measurement.

[TS 38.331, clause 5.3.2.3]

1>	if in RRC_INACTIVE, for each of the PagingRecord, if any, included in the Paging message, or

1>	if in RRC_INACTIVE, for the PagingRecord, if any, included in the UuMessageTransferSidelink message received from the connected L2 U2N Relay UE:

2>	if the ue-Identity included in the PagingRecord matches the UE's stored fullI-RNTI:

...

3>	else if mt-SDT indication was included in the Paging message and if the conditions for initiating SDT for a resume procedure initiated in response to RAN paging according to 5.3.13.1b are fulfilled:

4>	if pagingGroupList was not included in the Paging message; or

4>	if pagingGroupList was included in the Paging message but the UE has not joined any MBS session(s) indicated by the TMGI(s) included in the pagingGroupList; or

4>	if pagingGroupList was included in the Paging message, all the MBS session(s) indicated by the TMGI(s) included in the pagingGroupList that the UE has joined are configured to be received in RRC_INACTIVE, and inactiveReceptionAllowed was included for all these MBS session(s):

5>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mt-SDT;

NOTE 1a:	If a UE receives a Paging message including mt-SDT indication and inactiveReceptionAllowed indications for all the multicast session(s) the UE has joined and the UE initiates RRC connection resume, the UE starts monitoring the corresponding G-RNTI(s), if configured, and if multicast MCCH is present, the UE starts monitoring the Multicast MCCH-RNTI and acquires the MBSMulticastConfiguration message on multicast MCCH.

4>	else:

5>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mt-Access;

NOTE 2:	If both conditions for initiating MT-SDT and MO-SDT according to 5.3.13.1b are fulfilled, UE may initiate RRC connection resumption procedure for MT-SDT or MO-SDT based on implementation.

[TS 38.331, clause 5.3.13.1b]

When requesting lower layers to check the conditions for initiating SDT, RRC indicates to lower layers whether the resume procedure is initiated for mobile originated or mobile terminated case.

A UE in RRC_INACTIVE initiates the resume procedure for SDT when all of the following conditions are fulfilled:

1>	for the resume procedure initiated by the upper layers (i.e. mobile originated case):

2>	SIB1 includes sdt-ConfigCommon; and

2>	sdt-Config is configured; and

2>	all the pending data in UL is mapped to the radio bearers configured for SDT; and

2>	for an (e)RedCap UE when RedCap-specific initial downlink BWP includes no CD-SSB, ncd-SSB-RedCapInitialBWP-SDT is configured; and

2>	lower layers indicate that conditions for initiating MO-SDT as specified in TS 38.321 [3] are fulfilled.

1>	for the resume procedure initiated in response to RAN paging (i.e. mobile terminated case):

2>	lower layers indicate that conditions for initiating MT-SDT as specified in TS 38.321 [3] are fulfilled.

NOTE:	How the UE determines that all pending data in UL is mapped to radio bearers configured for SDT is left to UE implementation.

7.1.1.13.6.3	Test description

7.1.1.13.6.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

UE:

-	None.

Preamble:

-	The UE is in state 3N-A and Test Mode Activated according to TS 38.508-1 [4] Table 4.4A.2-3 with UE test loop mode A is established except that set to return no data in uplink.

7.1.1.13.6.3.2	Test procedure sequence

Table 7.1.1.13.6.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a RRCRelease message including sdt-Config-r17 in suspendConfig. | <-- | NR RRC: RRCRelease | - | - |
| 2 | The SS transmits a Paging message including mt-SDT indication. | <-- | NR RRC: Paging | - | - |
| 3 | Check: Does the UE transmit MSGA using a preamble outside of the SmallData FeatureCombinationPreambles, with the associated PUSCH resource containing a RRCResumeRequest message with resumeCause set to mt-SDT? | --> | MAC PDU ( NR RRC: RRCResumeRequest) | 1 | P |
| 4 | The SS transmits a MSGB including a successRAR MAC subPDU containing matching Contention Resolution Identity, C-RNTI and Timing Advance Command. | <-- | MAC PDU(successRAR) | - | - |
| 5 | SS transmits a RLC PDU in a MAC PDU on the DRB configured with SDT. | <-- | MAC PDU | - | - |
| 6 | Check: Does the UE transmit an HARQ ACK on PUCCH? | --> | HARQ ACK | 2 | P |
| 7 | The SS transmits an RRCResume message to bring UE to RRC_CONNECTED State. | <-- | NR RRC: RRCResume | - | - |
| 8 | The UE transmits an RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 9 | The SS changes the parameter ‘mt-SDT-RSRP-Threshold-r18’ in SIB1 of NR Cell 1 to 76 and starts broadcasting updated SIB1. Note: This value should result in meeting condition ‘RSRP is below the configured mt-SDT-RSRP-Threshold’ | - | - | - | - |
| 10 | The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. | <-- | PDCCH (DCI 1_0): Short Message | - | - |
| 11 | Wait for 2.1* modification period second for the UE to receive new system information. (Note 1) | - | - | - | - |
| 12 | The SS transmits a RRCRelease message including sdt-Config-r17 in suspendConfig. | <-- | NR RRC: RRCRelease | - | - |
| 13 | The SS transmits a Paging message including mt-SDT indication. | <-- | NR RRC: Paging | - | - |
| 14 | Check: Does the UE transmit MSGA using a preamble outside of the SmallData FeatureCombinationPreambles, with the associated PUSCH resource containing a RRCResumeRequest message with resumeCause set to mt-Access? | --> | MAC PDU ( NR RRC: RRCResumeRequest) | 3 | P |
| 15 | The SS transmits a MSGB including a successRAR MAC subPDU containing matching Contention Resolution Identity, C-RNTI and Timing Advance Command. | <-- | MAC PDU(successRAR) | - | - |
| 16 | The SS transmits a RRCResume message. | <-- | NR RRC: RRCResume | - | - |
| 17 | The UE transmits a RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 18 | The SS transmits a RRCRelease message | <-- | NR RRC: RRCRelease | - | - |
| Note 1: The modification period, expressed in number of radio frames = modificationPeriodCoeff * defaultPagingCycle. |  |  |  |  |  |

7.1.1.13.6.3.3	Specific message contents

Table 7.1.1.13.6.3.3-1: SIB1 (preamble and step 9, Table 7.1.1.13.6.3.2-1)

| Derivation path: TS 38.508-1 [4] Table 4.6.1-28 with condition MT-SDT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| servingCellConfigCommon | ServingCellConfigCommonSIB | Table 7.1.1.13.6.3.3-2 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| mt-SDT-ConfigCommonSIB-r18 SEQUENCE { |  |  |  |
| mt-SDT-RSRP-Threshold-r18 | 66 (-90dBm) |  | Preamble |
|  | 76 (-80dBm) |  | Step 9 |
| sdt-LogicalChannelSR-DelayTimer-r18 | sf512 |  |  |
| t319a-r18 | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.6.3.3-2: ServingCellConfigCommon SIB(7.1.1.13.6.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon SIB::= SEQUENCE { |  |  |  |
| downlinkConfigCommon | DownlinkConfigCommonSIB with condition SDT | TS 38.508-1 [4] Table 4.6.3-53 |  |
| uplinkConfigCommon | UplinkConfigCommonSIB | Table 7.1.1.13.6.3.3-5 |  |
| } |  |  |  |

Table 7.1.1.13.6.3.3-3: Void

Table 7.1.1.13.6.3.3-4: Void

Table 7.1.1.13.6.3.3-5: UplinkConfigCommonSIB (7.1.1.13.6.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-201 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkConfigCommonSIB ::= SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkCommon | Table 7.1.1.13.6.3.3-6 |  |
| } |  |  |  |

Table 7.1.1.13.6.3.3-6: BWP-UplinkCommon (Table 7.1.1.13.6.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkCommon ::= SEQUENCE { |  |  |  |
| msgA-ConfigCommon-r16 CHOICE { |  |  |  |
| setup | MsgA-ConfigCommon-r16 | TS 38.508-1 [4] Table 4.6.3-81A |  |
| } |  |  |  |
| AdditionalRACH-ConfigList-r17 SEQUENCE { |  |  |  |
| AdditionalRACH-Config-r17 SEQUENCE { |  |  |  |
| rach-ConfigCommon-r17 | Not present |  |  |
| msgA-ConfigCommon-r17 | MsgA-ConfigCommon-r16 | Table 7.1.1.13.6.3.3-7 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.6.3.3-7: MsgA-ConfigCommon-r16 (Table 7.1.1.13.6.3.3-6)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-81A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MsgA-ConfigCommon-r16 :: = SEQUENCE { |  |  |  |
| rach-ConfigCommonTwoStepRA-r16 | RACH-ConfigCommonTwoStepRA-r16 | Table 7.1.1.13.6.3.3-8 |  |
| } |  |  |  |

Table 7.1.1.13.6.3.3-8: RACH-ConfigCommonTwoStepRA-r16 (Table 7.1.1.13.6.3.3-7)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-128A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommonTwoStepRA-r16 ::= SEQUENCE { |  |  |  |
| featureCombinationPreamblesList-r17 SEQUENCE { |  |  |  |
| FeatureCombinationPreambles-r17 | FeatureCombinationPreambles | Table 7.1.1.13.6.3.3-9 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.6.3.3-9: FeatureCombinationPreambles (Table 7.1.1.13.6.3.3-8)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-56E |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FeatureCombinationPreambles-r17 ::= SEQUENCE { |  |  |  |
| featureCombination-r17 ::= SEQUENCE { |  |  |  |
| smallData-r17 | true |  |  |
| } |  |  |  |
| startPreambleForThisPartition-r17 | 8 | Randomly selected |  |
| numberOfPreamblesPerSSB-ForThisPartition-r17 | 12 |  |  |
| ssb-SharedRO-MaskIndex-r17 | Not present |  |  |
| groupBconfigured-r17 | Not present |  |  |
| separateMsgA-PUSCH-Config-r17 | MsgA-PUSCH-Config | Table 7.1.1.13.6.3.3-10 |  |
| msgA-RSRP-Threshold-r17 | 57 | -100 dBm |  |
| rsrp-ThresholdSSB-r17 | RSRP-Range | TS 38.508-1 [4] table 4.6.3-152 |  |
| deltaPreamble-r17 | Not present |  |  |
| } |  |  |  |

Table 7.1.1.13.6.3.3-10: MsgA-PUSCH-Config (Table 7.1.1.13.6.3.3-9)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-81B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MsgA-PUSCH-Config-r16 ::= SEQUENCE { |  |  |  |
| msgA-PUSCH-ResourceGroupA-r16 SEQUENCE { |  |  |  |
| msgA-MCS-r16 | 1 |  |  |
| nrofPRBs-PerMsgA-PO-r16 | 15 |  |  |
| } |  |  |  |

Table 7.1.1.13.6.3.3-11: RRCRelease (Steps 1, 12 Table 7.1.1.13.6.3.2-1)

| Derivation Path: TS 38.508-1 [4] table 4.6.1-16 with condition NR_RRC_INACTIVE and MT_SDT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |
| suspendConfig SEQUENCE { |  |  |  |
| sdt-Config-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sdt-DRB-List-r17 SEQUENCE (SIZE (0..maxDRB)) OF DRB-Identity { | 1 entry |  |  |
| DRB-Identity[1] | DRB-Identity using condition DRBj | Entry 1 j is the ID of the DRB established during the preamble which is allocated according to internal TTCN mapping |  |
| } |  |  |  |
| sdt-SRB2-Indication-r17 | Not present |  |  |
| sdt-MAC-PHY-CG-Config-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.6.3.3-12: Paging (Steps 2, 13 Table 7.1.1.13.6.3.2-1)

Derivation Path: TS 38.508-1 [4] table 4.6.1-9 with condition NR_RRC_RESUME and MT_SDT

Table 7.1.1.13.6.3.3-13: RRCResumeRequest (Steps 3, 14 Table 7.1.1.13.6.3.2-1)

| Derivation Path: TS 38.508-1 [4] table 4.6.1-19 with condition MT_SDT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCResumeRequest ::= SEQUENCE { |  |  |  |
| rrcResumeRequest SEQUENCE { |  |  |  |  |  |
| resumeCause | ResumeCause | Table 7.1.1.13.6.3.3-14 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.6.3.3-14: ResumeCause (Step 3, Step 14, Table 7.1.1.13.6.3.3-13)

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ResumeCause | mt-SDT-v1810 |  | Step 3 |
|  | mt-Access |  | Step 14 |

##### 7.1.1.13.7	RA Based MT-SDT / 4-step RACH/ Success

7.1.1.13.7.1	Test Purpose (TP)

(1)

with { UE in NR RRC_INACTIVE state and SDT-CG-Config is not configured and Random Access resources for RA-SDT are configured }

ensure that {

when { UE receives paging message with mt-SDT indication included and RSRP is above the configured mt-SDT-RSRP-Threshold }

then { UE shall initiate 4-step RA based mt-SDT procedure using legacy Random Access resources with resumeCause set to mt-SDT in RRC connection resumption procedure }

}

(2)

with { UE in NR RRC_INACTIVE state and SDT-CG-Config is configured and Random Access resources for RA-SDT are configured }

ensure that {

when { UE initiates RA based MT-SDT procedure }

then { UE is successfully able to receive subsequent SDT data }

}

(3)

with { UE in NR RRC_INACTIVE state and SDT-CG-Config is not configured and Random Access resources for RA-SDT are configured }
ensure that {
  when { UE receives paging message with mt-SDT indication included and RSRP is below the configured mt-SDT-RSRP-Threshold }
    then { UE shall not initiate RA based SDT procedure and starts normal RRC Resume procedure resumeCause set to mt-access in RRC connection resumption procedure }
            }

7.1.1.13.7.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 38.321, clause 5.1.1b, 5.1.1c & 5.27, 3GPP TS 38.331, clause 5.3.2.3 & 5.3.13.1b. Unless otherwise stated these are Rel-18 requirements.

[TS 38.321, clause 5.1.1b]

The MAC entity shall:

1>	if the BWP selected for Random Access procedure is configured with both set(s) of Random Access resources with MSG3 repetition indication and set(s) of Random Access resources without MSG3 repetition indication and the RSRP of the downlink pathloss reference is less than rsrp-ThresholdMsg3; or

1>	if the BWP selected for Random Access procedure is only configured with the set(s) of Random Access resources with MSG3 repetition indication:

2>	assume MSG3 repetition is applicable for the current Random Access procedure.

1>	else:

2>	assume MSG3 repetition is not applicable for the current Random Access procedure.

NOTE 1:	Void.

1>	if contention-free Random Access Resources have not been provided for this Random Access procedure and one or more of the features including RedCap and/or a specific NSAG(s) and/or SDT and/or MSG3 repetition is applicable for this Random Access procedure:

NOTE 2: The applicability of SDT is determined by MAC entity according to clause 5.27. The applicability of specific NSAG(s) is determined by upper layers when the Random Access procedure is initiated. The applicability of RedCap is also determined by upper layers when Random Access procedure is initiated and it is applicable to the Random Access procedures initiated by PDCCH orders and any Random Access procedure initiated by the MAC entity.

2>	if none of the sets of Random Access resources are available for any feature applicable to the current Random Access procedure (as specified in clause 5.1.1c):

3>	select the set(s) of Random Access resources that are not associated with any feature indication (as specified in clause 5.1.1c) for this Random Access procedure.

2>	else if there is one set of Random Access resources available which can be used for indicating all features triggering this Random Access procedure:

3>	select this set of Random Access resources for this Random Access procedure.

2>	else (i.e. there are one or more sets of Random Access resources available that are configured with indication(s) for a subset of all features triggering this Random Access procedure):

3>	select a set of Random Access resources from the available set(s) of Random Access resources based on the priority order indicated by upper layers as specified in clause 5.1.1d for this Random Access Procedure.

1>	else if contention-free Random Access Resources have been provided for this Random Access procedure and RedCap is applicable for the current Random Access procedure and there is one set of Random Access resources available that is only configured with RedCap indication:

2>	select this set of Random Access resources for this Random Access procedure.

1>	else:

2>	select the set of Random Access resources that are not associated with any feature indication (as specified in clause 5.1.1c) for the current Random Access procedure.

[TS 38.321, clause 5.1.1c]

The MAC entity shall for each set of configured Random Access resources:

1>	if eRedCap is set to true for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for a Random Access procedure for which eRedCap is not applicable.

1>	if redCap is set to true for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for a Random Access procedure for which RedCap is not applicable.

1>	if smallData is set to true for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for the Random Access procedure which is not triggered for RA-SDT by MO-SDT as specified in TS 38.331 [5].

1>	if NSAG-List is configured for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for the Random Access procedure unless it is triggered for any one of the NSAG-ID(s) in the NSAG-List.

1>	if msg3-Repetitions is set to true for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for the Random Access procedure if Msg3 repetition is not applicable.

1>	if msg1-Repetitions is set to true for a set of Random Access resources:

2>	if Msg1 repetition is not applicable to the current Random Access procedure; or

2>	if the set of Random Access resources is not associated with any of the Msg1 repetition number that is applicable to the current Random Access procedure:

3>	consider the set of Random Access resources as not available for the Random Access procedure.

1>	if a set of Random Access resources is not configured with FeatureCombination:

2>	consider the set of Random Access resources to not associated with any feature.

[TS 38.321, clause 5.27]

The MAC entity may be configured by RRC with SDT and the SDT procedure may be initiated by RRC layer for MO-SDT or MT-SDT

. The SDT procedure initiated for MO-SDT can be performed either by Random Access procedure with 2-step RA type or 4-step RA type (i.e., RA-SDT) or by configured grant Type 1 (i.e., CG-SDT). The SDT procedure initiated for MT-SDT can be performed either by Random Access procedure with 2-step RA type or 4-step RA type (i.e., RA-SDT is not applicable as specified in clause 5.1.1b) or by configured grant Type 1 (i.e., CG-SDT).

RRC configures the following parameters for SDT procedure:

-	sdt-DataVolumeThreshold: data volume threshold for the UE to determine whether to perform SDT procedure initiated for MO-SDT;

-	sdt-RSRP-Threshold: RSRP threshold for UE to determine whether to perform SDT procedure initiated for MO-SDT;

-	mt-SDT-RSRP-Threshold: RSRP threshold for UE to determine whether to perform SDT procedure initiated for MT-SDT;

-	cg-SDT-RSRP-ThresholdSSB: an RSRP threshold configured for SSB selection for CG-SDT;

-	cg-MT-SDT-MaxDurationToNextCG-Occasion: time threshold which is used by the UE to determine whether to perform CG-SDT for MT-SDT;

-	cg-SDT-MaxDurationToNextCG-Occasion: time threshold configured per logical channel which is used by the UE to determine whether to perform CG-SDT for MO-SDT;

-	sdt-BeamFailureRecoveryProhibitTimer: prohibit timer to avoid frequent triggering of Random Access procedure due to beam failure recovery during RA-SDT procedure or during MT-SDT procedure initiated by Random Access procedure.

The following UE variable is used for the SDT procedure:

-	MAX_DURATION_TO_NEXT_CG_OCCASION;

-	RSRP_THRESHOLD.

The MAC entity shall, if initiated by the upper layers for SDT procedure:

1>	if SDT procedure is initiated for MO-SDT as specified in TS 38.331 [5]:

2>	set the MAX_DURATION_TO_NEXT_CG_OCCASION to the shortest value of cg-SDT-MaxDurationToNextCG-Occasion, if configured, among all the logical channels configured with this parameter by upper layer and having data for transmission;

2>	set the RSRP_THRESHOLD to the value of sdt-RSRP-Threshold, if configured.

1>	else if SDT procedure is initiated for MT-SDT as specified in TS 38.331 [5]:

2>	set the MAX_DURATION_TO_NEXT_CG_OCCASION to the value of cg-MT-SDT-MaxDurationToNextCG-Occasion, if configured;

2>	if mt-SDT-RSRP-Threshold is configured:

3>	set the RSRP_THRESHOLD to the value of mt-SDT-RSRP-Threshold.

2>	else if sdt-RSRP-Threshold is configured:

3>	set the RSRP_THRESHOLD to the value of sdt-RSRP-Threshold.

1>	if the SDT procedure is initiated for MO-SDT as specified in TS 38.331 [5], and the data volume of the pending UL data across all RBs configured for SDT is less than or equal to sdt-DataVolumeThreshold, or if the SDT procedure is initiated for MT-SDT as specified in TS 38.331 [5]; and

NOTE 1:	For SDT procedure, the MAC entity also considers the suspended RBs configured with SDT for data volume calculation. It is up to the UE's implementation how the UE calculates the data volume for the suspended RBs. Size of the CCCH message is not considered for data volume calculation

1>	if the RSRP of the downlink pathloss reference is higher than RSRP_THRESHOLD or if RSRP_THRESHOLD is not set:

2>	if the Serving Cell is configured with supplementary uplink as specified in TS 38.331 [5]; and

2>	if the RSRP of the downlink pathloss reference is less than rsrp-ThresholdSSB-SUL:

3>	select the SUL carrier.

2>	else:

3>	select the NUL carrier.

2>	if CG-SDT is configured on the selected UL carrier, and TA for CG-SDT is valid according to clause 5.27.2 in the first available CG occasion for initial CG-SDT transmission with CCCH message according to clause 5.8.2; and

2>	if the SDT procedure is initiated for MO-SDT as specified in TS 38.331 [5], and, for each RB having data available for transmission, configuredGrantType1Allowed, if configured for CG-SDT, is configured with value true for the corresponding logical channel, or if the SDT procedure is initiated for MT-SDT as specified in TS 38.331 [5]; and

2>	if at least one SSB configured for CG-SDT with SS-RSRP above cg-SDT-RSRP-ThresholdSSB is available, and if either the time gap between the initiation of the SDT procedure and first available CG occasion for initial CG-SDT transmission with CCCH message according to clause 5.8.2 is less than MAX_DURATION_TO_NEXT_CG_OCCASION, or if the MAX_DURATION_TO_NEXT_CG_OCCASION is not set:

3>	indicate to the upper layers that the conditions for initiating SDT procedure are fulfilled;

3>	perform CG-SDT procedure on the selected UL carrier according to clause 5.8.2.

2>	else if a set of Random Access resources for RA-SDT is configured and can be selected according to clause 5.1.1b on the selected UL carrier on the BWP configured by initialUplinkBWP-RedCap, if configured for an (e)RedCap UE; otherwise, on the BWP configured by initialUplinkBWP; or

2>	if the SDT procedure is initiated for MT-SDT as specified in TS 38.331 [5]:

3>	if cg-SDT-TimeAlignmentTimer is running, consider cg-SDT-TimeAlignmentTimer as expired and perform the corresponding actions in clause 5.2;

3>	indicate to the upper layers that the conditions for initiating SDT procedure are fulfilled.

2>	else:

3>	indicate to the upper layers that the conditions for initiating SDT procedure are not fulfilled.

1>	else:

2>	indicate to the upper layers that the conditions for initiating SDT procedure are not fulfilled.

If Random Access procedure is selected above for SDT procedure initiated for MO-SDT or MT-SDT and after the Random Access procedure is successfully completed (see clause 5.1.6), the UE monitors PDCCH addressed to C-RNTI received in random access response until the SDT procedure is terminated. If CG-SDT is selected above and after the initial transmission for CG-SDT is performed, the UE monitors PDCCH addressed to C-RNTI as stored in UE Inactive AS context as specified in TS 38.331 [5] and CS-RNTI until the SDT procedure is terminated.

The MAC entity shall:

1>	if sdt-BeamFailureRecoveryProhibitTimer is configured and not running; and

1>	if RA-SDT procedure for MO-SDT or MT-SDT procedure initiated by Random Access procedure is ongoing:

2>	if SS-RSRP of the SSB selected in the last successfully completed Random Access procedure during ongoing RA-SDT procedure or ongoing MT-SDT procedure initiated by Random Access procedure is less than rsrp-ThresholdSSB:

3>	start the sdt-BeamFailureRecoveryProhibitTimer;

3>	initiate a Random Access procedure (see clause 5.1).

NOTE 1a:	It is up to UE implementation when to measure SSBs during ongoing RA-SDT procedure or ongoing MT-SDT procedure initiated by Random Access procedure. Based on implementation the UE need not initiate the Random Access procedure if there is no SSB with SS-RSRP above rsrp-ThresholdSSB. The UE uses rsrp-ThresholdSSB in Random Access configuration selected by the UE when RA-SDT or MT-SDT procedure was initiated.

NOTE 2:	When the UE determines if there is an SSB with SS-RSRP above cg-SDT-RSRP-ThresholdSSB or less than rsrp-ThresholdSSB, the UE uses the latest unfiltered L1-RSRP measurement.

[TS 38.331, clause 5.3.2.3]

1>	if in RRC_INACTIVE, for each of the PagingRecord, if any, included in the Paging message, or

1>	if in RRC_INACTIVE, for the PagingRecord, if any, included in the UuMessageTransferSidelink message received from the connected L2 U2N Relay UE:

2>	if the ue-Identity included in the PagingRecord matches the UE's stored fullI-RNTI:

…

3>	else if mt-SDT indication was included in the Paging message and if the conditions for initiating SDT for a resume procedure initiated in response to RAN paging according to 5.3.13.1b are fulfilled:

4>	if pagingGroupList was not included in the Paging message; or:

4>	if pagingGroupList was included in the Paging message but the UE has not joined any MBS session(s) indicated by the TMGI(s) included in the pagingGroupList or:

4>	if pagingGroupList was included in the Paging message, the UE is configured to receive MBS multicast in RRC_INACTIVE, and inactiveReceptionAllowed was included for all the MBS session(s) indicated by the TMGI(s) that the UE has joined:

5>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mt-SDT:

4>	else:

5>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mt-Access;

3>	else:

4>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mt-Access;

NOTE 2:	If both conditions for initiating MT-SDT and MO-SDT according to 5.3.13.1b are fulfilled, UE may initiate RRC connection resumption procedure for MT-SDT or MO-SDT based on implementation.

[TS 38.331, clause 5.3.13.1b]

When requesting lower layers to check the conditions for initiating SDT, RRC indicates to lower layers whether the resume procedure is initiated for mobile originated or mobile terminated case.

A UE in RRC_INACTIVE initiates the resume procedure for SDT when all of the following conditions are fulfilled:

1>	for the resume procedure initiated by the upper layers (i.e. mobile originated case):

2>	SIB1 includes sdt-ConfigCommon; and

2>	sdt-Config is configured; and

2>	all the pending data in UL is mapped to the radio bearers configured for SDT; and

2>	for an (e)RedCap UE when RedCap-specific initial downlink BWP includes no CD-SSB, ncd-SSB-RedCapInitialBWP-SDT is configured; and

2>	lower layers indicate that conditions for initiating MO-SDT as specified in TS 38.321 [3] are fulfilled.

1>	for the resume procedure initiated in response to RAN paging (i.e. mobile terminated case):

2>	lower layers indicate that conditions for initiating MT-SDT as specified in TS 38.321 [3] are fulfilled.

NOTE:	How the UE determines that all pending data in UL is mapped to radio bearers configured for SDT is left to UE implementation.

7.1.1.13.7.3	Test description

7.1.1.13.7.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

UE:

-	None.

Preamble:

-	The UE is in state 3N-A and Test Mode Activated according to TS 38.508-1 [4] Table 4.4A.2-3 with UE test loop mode A is established except that set to return no data in uplink.

7.1.1.13.7.3.2	Test procedure sequence

Table 7.1.1.13.7.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a RRCRelease message including sdt-Config-r17 in suspendConfig. | <-- | NR RRC: RRCRelease | - | - |
| 2 | The SS transmits a Paging message including mt-SDT indication. | <-- | NR RRC: Paging | - | - |
| 3 | Check: Does the UE transmit a preamble on PRACH using a preamble outside of the SmallData FeatureCombinationPreambles? | --> | PRACH Preamble | 1 | P |
| 4 | The SS transmits Random Access Response with RAPID corresponding to the transmitted Preamble in step 3, including TC-RNTI and not including Backoff Indicator subheader. | <-- | Random Access Response | - | - |
| 5 | Check: Does the UE transmit a MAC PDU containing an RRCResumeRequest message with resumeCause set to mt-SDT? | --> | MAC PDU ( NR RRC: RRCResumeRequest) | 1 | P |
| 6 | The SS schedules PDCCH transmission addressed to TC-RNTI to transmit a valid MAC PDU containing ‘UE Contention Resolution Identity’ MAC control element with matched Contention Resolution Identity’. | <-- | MAC PDU (UE Contention Resolution Identity MAC CE) | - | - |
| 7 | SS transmits a RLC PDU in a MAC PDU on the DRB configured with SDT. | <-- | MAC PDU |  |  |
| 8 | Check: Does the UE transmit an HARQ ACK on PUCCH? | --> | HARQ ACK | 2 | P |
| 9 | The SS transmits an RRCResume message to bring UE to RRC_CONNECTED State. | <-- | NR RRC: RRCResume | - | - |
| 10 | The UE transmits an RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 11 | The SS changes the parameter ‘sdt-RSRP-Threshold-r17’ in SIB1 of NR Cell 1 to 76 and starts broadcasting updated SIB1. Note: This value should result in meeting condition ‘RSRP is below the configured mt-SDT-RSRP-Threshold’ | - | - | - | - |
| 12 | The SS transmits a RRCRelease message including sdt-Config-r17 in suspendConfig. | <-- | NR RRC: RRCRelease | - | - |
| 13 | The SS transmits a Paging message including mt-SDT indication. | <-- | NR RRC: Paging | - | - |
| 14 | The UE transmits a preamble on PRACH using a preamble outside of the SmallData FeatureCombinationPreambles | --> | PRACH Preamble | - | - |
| 15 | The SS transmits Random Access Response with RAPID corresponding to the transmitted Preamble in step 14, including TC-RNTI and not including Backoff Indicator subheader. | <-- | Random Access Response | - | - |
| 16 | Check: Does the UE transmit a MAC PDU containing an RRCResumeRequest message with resumeCause set to mt-Access? | --> | MAC PDU ( NR RRC: RRCResumeRequest) | 3 | P |
| 18 | The SS schedules PDCCH transmission addressed to TC-RNTI to transmit a valid MAC PDU containing ‘UE Contention Resolution Identity’ MAC control element with matched ‘Contention Resolution Identity’. | <-- | MAC PDU (UE Contention Resolution Identity MAC CE) | - | - |
| 19 | The SS transmits a RRCResume message. | <-- | NR RRC: RRCResume | - | - |
| 20 | The UE transmits a RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 21 | The SS transmits a RRCRelease message | <-- | NR RRC: RRCRelease | - | - |

7.1.1.13.7.3.3	Specific message contents

Table 7.1.1.13.7.3.3-1: CLOSE UE TEST LOOP (Steps 14 and preamble Table 7.1.1.13.7.3.2-1)

Derivation Path: TS 36.508 [7] table 4.7A-3 condition UE test loop mode B

Table 7.1.1.13.7.3.3-2: SIB1 (preamble and step 12, Table 7.1.1.13.7.3.2-1)

| Derivation path: TS 38.508-1 [4] Table 4.6.1-28 with condition MT-SDT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| servingCellConfigCommon | ServingCellConfigCommon | Table 7.1.1.13.7.3.3-3 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| mt-SDT-ConfigCommonSIB-r18SEQUENCE { |  |  |  |
| mt-SDT-RSRP-Threshold-r18 | 66 (-90dBm) |  | Preamble |
|  | 76 (-80dBm) |  | Step 11 |
| sdt-LogicalChannelSR-DelayTimer-r18 | sf512 |  |  |
| t319a-r18 | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.7.3.3-3: ServingCellConfigCommon (7.1.1.13.7.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| downlinkConfigCommon | DownlinkConfigCommon | Table 7.1.1.13.7.3.3-7 |  |
| uplinkConfigCommon SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkCommon | Table 7.1.1.13.7.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.7.3.3-4: BWP-UplinkCommon (Table 7.1.1.13.7.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkCommon ::= SEQUENCE { |  |  |  |
| AdditionalRACH-ConfigList-r17 SEQUENCE (SIZE(1..maxAdditionalRACH-r17)) OF AdditionalRACH-Config-r17 { | 1 entry |  |  |
| AdditionalRACH-Config-r17[1] SEQUENCE { |  | Entry 1 |  |
| rach-ConfigCommon-r17 | RACH-ConfigCommon | Table 7.1.1.13.7.3.3-5 |  |
| msgA-ConfigCommon-r17 | Not present |  |  |
| ) |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.7.3.3-5: RACH-ConfigCommon (Table 7.1.1.13.7.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-128 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon::= SEQUENCE { |  |  |  |
| featureCombinationPreamblesList-r17 SEQUENCE { |  |  |  |
| FeatureCombinationPreambles-r17 | FeatureCombinationPreambles | Table 7.1.1.13.7.3.3-6 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.7.3.3-6: FeatureCombinationPreambles (Table 7.1.1.13.7.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-56E |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FeatureCombinationPreambles-r17 ::= SEQUENCE { |  |  |  |
| featureCombination-r17 ::= SEQUENCE { |  |  |  |
| smallData-r17 | true |  |  |
| } |  |  |  |
| startPreambleForThisPartition-r17 | 8 | Randomly selected |  |
| numberOfPreamblesPerSSB-ForThisPartition-r17 | 12 |  |  |
| ssb-SharedRO-MaskIndex-r17 | Not present |  |  |
| groupBconfigured-r17 | Not present |  |  |
| separateMsgA-PUSCH-Config-r17 | Not present |  |  |
| msgA-RSRP-Threshold-r17 | Not present |  |  |
| rsrp-ThresholdSSB-r17 | Not present |  |  |
| deltaPreamble-r17 | Not present |  |  |
| } |  |  |  |

Table 7.1.1.13.7.3.3-7: DownlinkConfigCommon

| Derivation Path: TS 38.508-1 [4] table 4.6.3-52 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DownlinkConfigCommon ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkCommon | Table 7.1.1.13.7.3.3-8 |  |
| } |  |  |  |

Table 7.1.1.13.7.3.3-8: BWP-DownlinkCommon

| Derivation Path: TS 38.508-1 [4] table 4.6.3-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkCommon ::= SEQUENCE { |  |  |  |
| pdcch-ConfigCommon CHOICE { |  |  |  |
| setup | PDCCH-ConfigCommon with condition SDT |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.7.3.3-9: RRCRelease (Steps 1, 13 Table 7.1.1.13.7.3.2-1 )

| Derivation Path: TS 38.508-1 [4] table 4.6.1-16 with condition NR_RRC_INACTIVE and SDT |  |  |  |
| --- | --- | --- | --- |
| RRCRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |
| suspendConfig SEQUENCE { |  |  |  |
| sdt-DRB-List-r17 SEQUENCE (SIZE (0..maxDRB)) OF DRB-Identity { | 1 entry |  |  |
| DRB-Identity[1] | DRB-Identity using condition DRBj | Entry 1 j is the ID of the DRB established during the preamble which is allocated according to internal TTCN mapping |  |
| } |  |  |  |
| sdt-SRB2-Indication-r17 | Not present |  |  |
| sdt-MAC-PHY-CG-Config-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 7.1.1.13.8	RA Based MT-SDT / 4-Step RACH /RACH resources instead of CG resources / Success

7.1.1.13.8.1	Test Purpose (TP)

(1)

with { UE in NR RRC_INACTIVE state and SDT-CG-Config-r17 is configured }

ensure that {

when { { UE receives paging message with mt-SDT indication included and RSRP is above the configured mt-SDT-RSRP-Threshold and at least one SSB configured for CG-SDT with SS-RSRP above cg-SDT-RSRP-ThresholdSSB and cg-SDT-TimeAlignmentTimer is running}

then { UE shall initiate CG based SDT procedure }

}

(2)

with { UE in NR RRC_INACTIVE state and SDT-CG-Config-r17 is configured }

ensure that {

when { { UE receives paging message with mt-SDT indication included and RSRP is above the configured mt-SDT-RSRP-Threshold and at least one SSB configured for CG-SDT with SS-RSRP above cg-SDT-RSRP-ThresholdSSB and cg-SDT-TimeAlignmentTimer expires }

then { UE shall initiate RA based SDT procedure }

}

(3)

with { UE in NR RRC_INACTIVE state and CG-SDT for MT-SDT is ongoing}

ensure that {

when { cg-SDT-TimeAlignmentTimer expires before receiving network response for the UL CG-SDT transmission with CCCH message }

then { UE considers ongoing CG-SDT procedure as terminated and performs the actions upon going to RRC_IDLE with release cause 'RRC Resume failure' }

}

(4)

with { UE in NR RRC_INACTIVE state and CG-SDT for MT-SDT is ongoing}

ensure that {

when { cg-SDT-TimeAlignmentTimer expires after receiving network response for the UL CG-SDT transmission with CCCH message }

then { UE does not consider ongoing CG-SDT procedure as terminated and UE shall perform uplink transmission using legacy Random Access }

}

7.1.1.13.8.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 38.321, clause 5.1.1b, 5.1.1c & 5.27, 3GPP TS 38.331, clause 5.3.2.3 & 5.3.13.1b. Unless otherwise stated these are Rel-18 requirements.

[TS 38.321, clause 5.2]

RRC configures the following parameters for the maintenance of UL time alignment:

…

-	cg-SDT-TimeAlignmentTimer which controls how long the MAC entity considers the uplink transmission for CG-SDT to be uplink time aligned;

-	inactivePosSRS-ValidityAreaTAT which controls how long the MAC entity considers Positioning SRS transmission in RRC_INACTIVE in clause 5.26 to be uplink time aligned when SRS positioning validity area is configured.

…

1>	when the cg-SDT-TimeAlignmentTimer expires:

2>	clear any configured uplink grants;

2>	if a PDCCH addressed to the MAC entity's C-RNTI after initial transmission for the CG-SDT with CCCH message has not been received:

3>	consider ongoing CG-SDT procedure as terminated;

3>	indicate the expiry of cg-SDT-TimeAlignmentTimer to the upper layer.

2>	flush all HARQ buffers;

2>	maintain NTA (defined in TS 38.211 [8]) of this TAG.

…

The MAC entity shall not perform any uplink transmission on a Serving Cell except the Random Access Preamble and MSGA transmission when the timeAlignmentTimer(s) associated with all TAG(s) to which this Serving Cell belongs is not running, CG-SDT procedure is not ongoing and SRS transmission in RRC_INACTIVE as in clause 5.26 is not ongoing. Furthermore, when the timeAlignmentTimer(s) associated with all PTAG(s) is not running, CG-SDT procedure is not ongoing and SRS transmission in RRC_INACTIVE as in clause 5.26 is not ongoing, the MAC entity shall not perform any uplink transmission on any Serving Cell except the Random Access Preamble and MSGA transmission on the SpCell. The MAC entity shall not perform any uplink transmission except the Random Access Preamble and MSGA transmission when the cg-SDT-TimeAlignmentTimer is not running during the ongoing CG-SDT procedure as triggered in clause 5.27 and the inactivePosSRS-TimeAlignmentTimer or inactivePosSRS-ValidityAreaTAT is not running. The MAC entity shall not perform any uplink transmission except the Random Access Preamble and MSGA transmission on a Serving Cell using TCI state(s) associated with a TAG for which the timeAlignmentTimer is not running.

[TS 38.321 clause 5.27.1]

The MAC entity shall, if initiated by the upper layers for SDT procedure:

1>	if SDT procedure is initiated for MO-SDT as specified in TS 38.331 [5]:

2>	set the MAX_DURATION_TO_NEXT_CG_OCCASION to the shortest value of cg-SDT-MaxDurationToNextCG-Occasion, if configured, among all the logical channels configured with this parameter by upper layer and having data for transmission;

2>	set the RSRP_THRESHOLD to the value of sdt-RSRP-Threshold, if configured.

1>	else if SDT procedure is initiated for MT-SDT as specified in TS 38.331 [5]:

2>	set the MAX_DURATION_TO_NEXT_CG_OCCASION to the value of cg-MT-SDT-MaxDurationToNextCG-Occasion, if configured;

2>	if mt-SDT-RSRP-Threshold is configured:

3>	set the RSRP_THRESHOLD to the value of mt-SDT-RSRP-Threshold.

2>	else if sdt-RSRP-Threshold is configured:

3>	set the RSRP_THRESHOLD to the value of sdt-RSRP-Threshold.

1>	if the SDT procedure is initiated for MO-SDT as specified in TS 38.331 [5], and the data volume of the pending UL data across all RBs configured for SDT is less than or equal to sdt-DataVolumeThreshold, or if the SDT procedure is initiated for MT-SDT as specified in TS 38.331 [5]; and

NOTE 1:	For SDT procedure, the MAC entity also considers the suspended RBs configured with SDT for data volume calculation. It is up to the UE's implementation how the UE calculates the data volume for the suspended RBs. Size of the CCCH message is not considered for data volume calculation

1>	if the RSRP of the downlink pathloss reference is higher than RSRP_THRESHOLD or if RSRP_THRESHOLD is not set:

2>	if the Serving Cell is configured with supplementary uplink as specified in TS 38.331 [5]; and

2>	if the RSRP of the downlink pathloss reference is less than rsrp-ThresholdSSB-SUL:

3>	select the SUL carrier.

2>	else:

3>	select the NUL carrier.

2>	if CG-SDT is configured on the selected UL carrier, and TA for CG-SDT is valid according to clause 5.27.2 in the first available CG occasion for initial CG-SDT transmission with CCCH message according to clause 5.8.2; and

2>	if the SDT procedure is initiated for MO-SDT as specified in TS 38.331 [5], and, for each RB having data available for transmission, configuredGrantType1Allowed, if configured for CG-SDT, is configured with value true for the corresponding logical channel, or if the SDT procedure is initiated for MT-SDT as specified in TS 38.331 [5]; and

2>	if at least one SSB configured for CG-SDT with SS-RSRP above cg-SDT-RSRP-ThresholdSSB is available, and if either the time gap between the initiation of the SDT procedure and first available CG occasion for initial CG-SDT transmission with CCCH message according to clause 5.8.2 is less than MAX_DURATION_TO_NEXT_CG_OCCASION, or if the MAX_DURATION_TO_NEXT_CG_OCCASION is not set:

3>	indicate to the upper layers that the conditions for initiating SDT procedure are fulfilled;

3>	perform CG-SDT procedure on the selected UL carrier according to clause 5.8.2.

2>	else if a set of Random Access resources for RA-SDT is configured and can be selected according to clause 5.1.1b on the selected UL carrier on the BWP configured by initialUplinkBWP-RedCap, if configured for an (e)RedCap UE; otherwise, on the BWP configured by initialUplinkBWP; or

2>	if the SDT procedure is initiated for MT-SDT as specified in TS 38.331 [5]:

3>	if cg-SDT-TimeAlignmentTimer is running, consider cg-SDT-TimeAlignmentTimer as expired and perform the corresponding actions in clause 5.2;

3>	indicate to the upper layers that the conditions for initiating SDT procedure are fulfilled.

2>	else:

3>	indicate to the upper layers that the conditions for initiating SDT procedure are not fulfilled.

1>	else:

2>	indicate to the upper layers that the conditions for initiating SDT procedure are not fulfilled.

If Random Access procedure is selected above for SDT procedure initiated for MO-SDT or MT-SDT and after the Random Access procedure is successfully completed (see clause 5.1.6), the UE monitors PDCCH addressed to C-RNTI received in random access response until the SDT procedure is terminated. If CG-SDT is selected above and after the initial transmission for CG-SDT is performed, the UE monitors PDCCH addressed to C-RNTI as stored in UE Inactive AS context as specified in TS 38.331 [5] and CS-RNTI until the SDT procedure is terminated.

The MAC entity shall:

1>	if sdt-BeamFailureRecoveryProhibitTimer is configured and not running; and

1>	if RA-SDT procedure for MO-SDT or MT-SDT procedure initiated by Random Access procedure is ongoing:

2>	if SS-RSRP of the SSB selected in the last successfully completed Random Access procedure during ongoing RA-SDT procedure or ongoing MT-SDT procedure initiated by Random Access procedure is less than rsrp-ThresholdSSB:

3>	start the sdt-BeamFailureRecoveryProhibitTimer;

3>	initiate a Random Access procedure (see clause 5.1).

NOTE 1a:	It is up to UE implementation when to measure SSBs during ongoing RA-SDT procedure or ongoing MT-SDT procedure initiated by Random Access procedure. Based on implementation the UE need not initiate the Random Access procedure if there is no SSB with SS-RSRP above rsrp-ThresholdSSB. The UE uses rsrp-ThresholdSSB in Random Access configuration selected by the UE when RA-SDT or MT-SDT procedure was initiated.

NOTE 2:	When the UE determines if there is an SSB with SS-RSRP above cg-SDT-RSRP-ThresholdSSB or less than rsrp-ThresholdSSB, the UE uses the latest unfiltered L1-RSRP measurement.

[TS 38.321 clause 5.27.1]

The MAC entity may be configured by RRC with SDT and the SDT procedure may be initiated by RRC layer for MO-SDT or MT-SDT

. The SDT procedure initiated for MO-SDT can be performed either by Random Access procedure with 2-step RA type or 4-step RA type (i.e., RA-SDT) or by configured grant Type 1 (i.e., CG-SDT). The SDT procedure initiated for MT-SDT can be performed either by Random Access procedure with 2-step RA type or 4-step RA type (i.e., RA-SDT is not applicable as specified in clause 5.1.1b) or by configured grant Type 1 (i.e., CG-SDT).

RRC configures the following parameters for SDT procedure:

-	sdt-DataVolumeThreshold: data volume threshold for the UE to determine whether to perform SDT procedure initiated for MO-SDT;

-	sdt-RSRP-Threshold: RSRP threshold for UE to determine whether to perform SDT procedure initiated for MO-SDT;

-	mt-SDT-RSRP-Threshold: RSRP threshold for UE to determine whether to perform SDT procedure initiated for MT-SDT;

-	cg-SDT-RSRP-ThresholdSSB: an RSRP threshold configured for SSB selection for CG-SDT;

-	cg-MT-SDT-MaxDurationToNextCG-Occasion: time threshold which is used by the UE to determine whether to perform CG-SDT for MT-SDT;

-	cg-SDT-MaxDurationToNextCG-Occasion: time threshold configured per logical channel which is used by the UE to determine whether to perform CG-SDT for MO-SDT;

-	sdt-BeamFailureRecoveryProhibitTimer: prohibit timer to avoid frequent triggering of Random Access procedure due to beam failure recovery during RA-SDT procedure or during MT-SDT procedure initiated by Random Access procedure.

The following UE variable is used for the SDT procedure:

-	MAX_DURATION_TO_NEXT_CG_OCCASION;

-	RSRP_THRESHOLD.

The MAC entity shall, if initiated by the upper layers for SDT procedure:

1>	if SDT procedure is initiated for MO-SDT as specified in TS 38.331 [5]:

2>	set the MAX_DURATION_TO_NEXT_CG_OCCASION to the shortest value of cg-SDT-MaxDurationToNextCG-Occasion, if configured, among all the logical channels configured with this parameter by upper layer and having data for transmission;

2>	set the RSRP_THRESHOLD to the value of sdt-RSRP-Threshold, if configured.

1>	else if SDT procedure is initiated for MT-SDT as specified in TS 38.331 [5]:

2>	set the MAX_DURATION_TO_NEXT_CG_OCCASION to the value of cg-MT-SDT-MaxDurationToNextCG-Occasion, if configured;

2>	if mt-SDT-RSRP-Threshold is configured:

3>	set the RSRP_THRESHOLD to the value of mt-SDT-RSRP-Threshold.

2>	else if sdt-RSRP-Threshold is configured:

3>	set the RSRP_THRESHOLD to the value of sdt-RSRP-Threshold.

1>	if the SDT procedure is initiated for MO-SDT as specified in TS 38.331 [5], and the data volume of the pending UL data across all RBs configured for SDT is less than or equal to sdt-DataVolumeThreshold, or if the SDT procedure is initiated for MT-SDT as specified in TS 38.331 [5]; and

NOTE 1:	For SDT procedure, the MAC entity also considers the suspended RBs configured with SDT for data volume calculation. It is up to the UE's implementation how the UE calculates the data volume for the suspended RBs. Size of the CCCH message is not considered for data volume calculation

1>	if the RSRP of the downlink pathloss reference is higher than RSRP_THRESHOLD or if RSRP_THRESHOLD is not set:

2>	if the Serving Cell is configured with supplementary uplink as specified in TS 38.331 [5]; and

2>	if the RSRP of the downlink pathloss reference is less than rsrp-ThresholdSSB-SUL:

3>	select the SUL carrier.

2>	else:

3>	select the NUL carrier.

2>	if CG-SDT is configured on the selected UL carrier, and TA for CG-SDT is valid according to clause 5.27.2 in the first available CG occasion for initial CG-SDT transmission with CCCH message according to clause 5.8.2; and

2>	if the SDT procedure is initiated for MO-SDT as specified in TS 38.331 [5], and, for each RB having data available for transmission, configuredGrantType1Allowed, if configured for CG-SDT, is configured with value true for the corresponding logical channel, or if the SDT procedure is initiated for MT-SDT as specified in TS 38.331 [5]; and

2>	if at least one SSB configured for CG-SDT with SS-RSRP above cg-SDT-RSRP-ThresholdSSB is available, and if either the time gap between the initiation of the SDT procedure and first available CG occasion for initial CG-SDT transmission with CCCH message according to clause 5.8.2 is less than MAX_DURATION_TO_NEXT_CG_OCCASION, or if the MAX_DURATION_TO_NEXT_CG_OCCASION is not set:

3>	indicate to the upper layers that the conditions for initiating SDT procedure are fulfilled;

3>	perform CG-SDT procedure on the selected UL carrier according to clause 5.8.2.

2>	else if a set of Random Access resources for RA-SDT is configured and can be selected according to clause 5.1.1b on the selected UL carrier on the BWP configured by initialUplinkBWP-RedCap, if configured for an (e)RedCap UE; otherwise, on the BWP configured by initialUplinkBWP; or

2>	if the SDT procedure is initiated for MT-SDT as specified in TS 38.331 [5]:

3>	if cg-SDT-TimeAlignmentTimer is running, consider cg-SDT-TimeAlignmentTimer as expired and perform the corresponding actions in clause 5.2;

3>	indicate to the upper layers that the conditions for initiating SDT procedure are fulfilled.

2>	else:

3>	indicate to the upper layers that the conditions for initiating SDT procedure are not fulfilled.

1>	else:

2>	indicate to the upper layers that the conditions for initiating SDT procedure are not fulfilled.

If Random Access procedure is selected above for SDT procedure initiated for MO-SDT or MT-SDT and after the Random Access procedure is successfully completed (see clause 5.1.6), the UE monitors PDCCH addressed to C-RNTI received in random access response until the SDT procedure is terminated. If CG-SDT is selected above and after the initial transmission for CG-SDT is performed, the UE monitors PDCCH addressed to C-RNTI as stored in UE Inactive AS context as specified in TS 38.331 [5] and CS-RNTI until the SDT procedure is terminated.

The MAC entity shall:

1>	if sdt-BeamFailureRecoveryProhibitTimer is configured and not running; and

1>	if RA-SDT procedure for MO-SDT or MT-SDT procedure initiated by Random Access procedure is ongoing:

2>	if SS-RSRP of the SSB selected in the last successfully completed Random Access procedure during ongoing RA-SDT procedure or ongoing MT-SDT procedure initiated by Random Access procedure is less than rsrp-ThresholdSSB:

3>	start the sdt-BeamFailureRecoveryProhibitTimer;

3>	initiate a Random Access procedure (see clause 5.1).

NOTE 1a:	It is up to UE implementation when to measure SSBs during ongoing RA-SDT procedure or ongoing MT-SDT procedure initiated by Random Access procedure. Based on implementation the UE need not initiate the Random Access procedure if there is no SSB with SS-RSRP above rsrp-ThresholdSSB. The UE uses rsrp-ThresholdSSB in Random Access configuration selected by the UE when RA-SDT or MT-SDT procedure was initiated.

NOTE 2:	When the UE determines if there is an SSB with SS-RSRP above cg-SDT-RSRP-ThresholdSSB or less than rsrp-ThresholdSSB, the UE uses the latest unfiltered L1-RSRP measurement.

[TS 38.321 clause 5.27.2]

The MAC entity shall, upon the reception of CG-SDT configuration:

1>	store the current RSRP of the downlink pathloss reference for TA validation as defined in TS 38.331 [5] clause 5.7.17.

The MAC entity shall consider the TA of the initial CG-SDT transmission with CCCH message to be valid when the following conditions are fulfilled:

1>	The RSRP values for the stored downlink pathloss reference and the current downlink pathloss reference are valid according to TS 38.133 [11]; and

1>	Compared to the stored downlink pathloss reference RSRP value, the current RSRP value of the downlink pathloss reference calculated as specified in TS 38.133 [11] has not increased/decreased by more than cg-SDT-RSRP-ChangeThreshold, if configured; and

1>	cg-SDT-TimeAlignmentTimer is running

[TS 38.331, clause 5.3.2.3]

1>	if in RRC_INACTIVE, for each of the PagingRecord, if any, included in the Paging message, or

1>	if in RRC_INACTIVE, for the PagingRecord, if any, included in the UuMessageTransferSidelink message received from the connected L2 U2N Relay UE:

2>	if the ue-Identity included in the PagingRecord matches the UE's stored fullI-RNTI:

…

3>	else if mt-SDT indication was included in the Paging message and if the conditions for initiating SDT for a resume procedure initiated in response to RAN paging according to 5.3.13.1b are fulfilled:

4>	if pagingGroupList was not included in the Paging message; or:

4>	if pagingGroupList was included in the Paging message but the UE has not joined any MBS session(s) indicated by the TMGI(s) included in the pagingGroupList or:

4>	if pagingGroupList was included in the Paging message, the UE is configured to receive MBS multicast in RRC_INACTIVE, and inactiveReceptionAllowed was included for all the MBS session(s) indicated by the TMGI(s) that the UE has joined:

5>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mt-SDT:

4>	else:

5>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mt-Access;

3>	else:

4>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mt-Access;

NOTE 2:	If both conditions for initiating MT-SDT and MO-SDT according to 5.3.13.1b are fulfilled, UE may initiate RRC connection resumption procedure for MT-SDT or MO-SDT based on implementation.

[TS 38.331, clause 5.3.13.1b]

When requesting lower layers to check the conditions for initiating SDT, RRC indicates to lower layers whether the resume procedure is initiated for mobile originated or mobile terminated case.

A UE in RRC_INACTIVE initiates the resume procedure for SDT when all of the following conditions are fulfilled:

1>	for the resume procedure initiated by the upper layers (i.e. mobile originated case):

2>	SIB1 includes sdt-ConfigCommon; and

2>	sdt-Config is configured; and

2>	all the pending data in UL is mapped to the radio bearers configured for SDT; and

2>	for an (e)RedCap UE when RedCap-specific initial downlink BWP includes no CD-SSB, ncd-SSB-RedCapInitialBWP-SDT is configured; and

2>	lower layers indicate that conditions for initiating MO-SDT as specified in TS 38.321 [3] are fulfilled.

1>	for the resume procedure initiated in response to RAN paging (i.e. mobile terminated case):

2>	lower layers indicate that conditions for initiating MT-SDT as specified in TS 38.321 [3] are fulfilled.

NOTE:	How the UE determines that all pending data in UL is mapped to radio bearers configured for SDT is left to UE implementation.

7.1.1.13.8.3	Test description

7.1.1.13.8.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

UE:

-	None.

Preamble:

-	The UE is in state 3N-A and Test Mode Activated according to TS 38.508-1 [4] Table 4.4A.2-3 with UE test loop mode A is established except that set to return no data in uplink.

7.1.1.13.8.3.2	Test procedure sequence

Table 7.1.1.13.8.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCRelease message including SDT-CG-Config-r17 in suspendConfig. The cg-SDT-TimeAlignmentTimer is configured to ms10240. | <-- | NR RRC: RRCRelease | - | - |
| 2 | The SS transmits a Paging message including mt-SDT indication. | <-- | NR RRC: Paging | - | - |
| 3 | Check: Does the UE transmit a MAC PDU containing an RRCResumeRequest message in a CG PUSCH occasion? | --> | MAC PDU ( NR RRC: RRCResumeRequest, ) | 1 | P |
| 4 | The SS waits 12s from step 1 to ensure that cg-SDT-TimeAlignmentTimer expired and then transmits Paging message including ng-5G-S-TMSI. (Note 1) | <-- | NR RRC: Paging | - | - |
| 5 | Check: Does the UE transmit an RRCSetupRequest message? | --> | NR RRC: RRCSetupRequest | 3 | P |
| 6-11 | Steps 3 to 8 of the NR RRC_CONNECTED procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 12 | The SS transmits an RRCRelease message including SDT-CG-Config-r17 in suspendConfig. The cg-SDT-TimeAlignmentTimer is configured to ms10240. | <-- | NR RRC: RRCRelease | - | - |
| 13 | The SS transmits a Paging message including mt-SDT indication. | <-- | NR RRC: Paging | - | - |
| 14 | Check: Does the UE transmit a MAC PDU containing an RRCResumeRequest message in a CG PUSCH occasion? | --> | MAC PDU ( NR RRC: RRCResumeRequest ) | 1 | P |
| 15 | The SS waits 12s from step 12 to ensure that cg-SDT-TimeAlignmentTimer expired and then transmits Paging message including matched fullI-RNTI. (Note 1) | <-- | NR RRC: Paging | - | - |
| 16 | Check: Does the UE transmit an RRCResumRequest message within 10s? | --> | NR RRC: RRCResumeRequest | 3 | F |
| 17-24 | The test steps 1 to 8 of generic test procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are performed on NR Cell 1. | - | - | - | - |
| 25 | The SS transmits an RRCRelease message including SDT-CG-Config-r17 in suspendConfig. The cg-SDT-TimeAlignmentTimer is configured to ms10240. | <-- | NR RRC: RRCRelease | - | - |
| 26 | The SS transmits a Paging message including mt-SDT indication. | <-- | NR RRC: Paging | - | - |
| 27 | Check: Does the UE transmit a MAC PDU containing an RRCResumeRequest message in a CG PUSCH occasion? | --> | MAC PDU ( NR RRC: RRCResumeRequest) | 1 | P |
| 28 | The SS waits 12s from step 25 to ensure that cg-SDT-TimeAlignmentTimer expired and then transmits a RLC PDU in a MAC PDU on the DRB configured with SDT. | <-- | MAC PDU | - | - |
| 29 | Check: Does the UE transmit a preamble on PRACH with ra-PreambleIndex outside of the SmallData FeatureCombinationPreambles (range from 0 to 7 for FR1 and from 0 to 3 for FR2)? | --> | PRACH Preamble | 4 | P |
| 30 | The SS transmits Random Access Response with RAPID corresponding to the transmitted Preamble in step 29, including TC-RNTI and not including Back off Indicator subheader. | <-- | Random Access Response | - | - |
| 31 | Check: Does the UE transmit an RRCResumeRequest message? | --> | MAC PDU ( NR RRC: RRCResumeRequest) | 4 | P |
| 32 | The SS schedules PDCCH transmission addressed to TC-RNTI to transmit a valid MAC PDU containing ‘UE Contention Resolution Identity’ MAC control element with matched ‘Contention Resolution Identity’. | <-- | MAC PDU (UE Contention Resolution Identity MAC CE and NR RRC: RRCResume) | - | - |
| 33 | The UE transmits an RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 34 | The SS transmits an RRCRelease message including SDT-CG-Config-r17 in suspendConfig. The cg-SDT-TimeAlignmentTimer is configured to ms10240. | <-- | NR RRC: RRCRelease | - | - |
| 35 | The SS waits 12s from step 34 to ensure that cg-SDT-TimeAlignmentTimer expired and then transmits Paging message including ng-5G-S-TMSI. (Note 1) | <-- | NR RRC: Paging | - | - |
| 36 | Check: Does the UE transmit a preamble on PRACH with ra-PreambleIndex outside of the SmallData FeatureCombinationPreambles (range from 0 to 7 for FR1 and from 0 to 3 for FR2)? | --> | PRACH Preamble | 2 | P |
| 37 | The SS transmits Random Access Response with RAPID corresponding to the transmitted Preamble in step 49a8, including TC-RNTI and not including Back off Indicator subheader. | <-- | Random Access Response | - | - |
| 38 | Check: The UE transmits a MAC PDU containing an RRCResumeRequest message? | --> | MAC PDU ( NR RRC: RRCResumeRequest,) | 2 | P |
| 39 | The SS schedules PDCCH transmission addressed to TC-RNTI to transmit a valid MAC PDU containing ‘UE Contention Resolution Identity’ MAC control element with matched ‘Contention Resolution Identity’. | <-- | MAC PDU (UE Contention Resolution Identity MAC CE) | - | - |
| 40 | The SS transmits a RRCRelease message | <-- | NR RRC: RRCRelease | - | - |
| Note 1: After cg-SDT-TimeAlignmentTimer expired, UE considers ongoing CG-SDT procedure as terminated and performs the actions upon going to RRC_IDLE. Therefore UE could receive CN Paging using 5G-S-TMSI and could not receive RAN Paging using fullI-RNTI. |  |  |  |  |  |

7.1.1.13.8.3.3	Specific message contents

Table 7.1.1.13.8.3.3-1: SIB1 (preamble Table 7.1.1.13.8.3.2-1)

| Derivation path: TS 38.508-1 [4] Table 4.6.1-28 with condition MT-SDT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| servingCellConfigCommon | ServingCellConfigCommon | Table 7.1.1.13.7.3.3-3 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| mt-SDT-ConfigCommonSIB-r18SEQUENCE { |  |  |  |
| mt-SDT-RSRP-Threshold-r18 | 66 (-90dBm) |  |  |
| sdt-LogicalChannelSR-DelayTimer-r18 | sf512 |  |  |
| t319a-r18 | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.8.3.3-2: ServingCellConfigCommon (7.1.1.13.8.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| downlinkConfigCommon | DownlinkConfigCommon | Table 7.1.1.13.8.3.3-6 |  |
| uplinkConfigCommon SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkCommon | Table 7.1.1.13.8.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.8.3.3-3: BWP-UplinkCommon (Table 7.1.1.13.8.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkCommon ::= SEQUENCE { |  |  |  |
| AdditionalRACH-ConfigList-r17 SEQUENCE (SIZE(1..maxAdditionalRACH-r17)) OF AdditionalRACH-Config-r17 { | 1 entry |  |  |
| AdditionalRACH-Config-r17[1] SEQUENCE { |  | Entry 1 |  |
| rach-ConfigCommon-r17 | RACH-ConfigCommon | Table 7.1.1.13.8.3.3-5 |  |
| msgA-ConfigCommon-r17 | Not present |  |  |
| ) |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.8.3.3-4: RACH-ConfigCommon (Table 7.1.1.13.8.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-128 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon::= SEQUENCE { |  |  |  |
| featureCombinationPreamblesList-r17 SEQUENCE { |  |  |  |
| FeatureCombinationPreambles-r17 | FeatureCombinationPreambles | Table 7.1.1.13.8.3.3-5 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.8.3.3-5: FeatureCombinationPreambles (Table 7.1.1.13.8.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-56E |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FeatureCombinationPreambles-r17 ::= SEQUENCE { |  |  |  |
| featureCombination-r17 ::= SEQUENCE { |  |  |  |
| smallData-r17 | true |  |  |
| } |  |  |  |
| startPreambleForThisPartition-r17 | 8 | Randomly selected |  |
| numberOfPreamblesPerSSB-ForThisPartition-r17 | 12 |  |  |
| ssb-SharedRO-MaskIndex-r17 | Not present |  |  |
| groupBconfigured-r17 | Not present |  |  |
| separateMsgA-PUSCH-Config-r17 | Not present |  |  |
| msgA-RSRP-Threshold-r17 | Not present |  |  |
| rsrp-ThresholdSSB-r17 | Not present |  |  |
| deltaPreamble-r17 | Not present |  |  |
| } |  |  |  |

Table 7.1.1.13.8.3.3-6: DownlinkConfigCommon

| Derivation Path: TS 38.508-1 [4] table 4.6.3-52 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DownlinkConfigCommon ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkCommon | Table 7.1.1.13.8.3.3-7 |  |
| } |  |  |  |

Table 7.1.1.13.8.3.3-7: BWP-DownlinkCommon

| Derivation Path: TS 38.508-1 [4] table 4.6.3-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkCommon ::= SEQUENCE { |  |  |  |
| pdcch-ConfigCommon CHOICE { |  |  |  |
| setup | PDCCH-ConfigCommon with condition SDT |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.8.3.3-8: RRCRelease (Step 1, 12, 25 & 34, Table 7.1.1.13.8.3.2-1 )

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-16 with condition NR_RRC_INACTIVE and MT-SDT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCRelease ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |
| suspendConfig SEQUENCE { |  |  |  |
| sdt-Config-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sdt-DRB-List-r17 SEQUENCE (SIZE (0..maxDRB)) OF DRB-Identity { | 1 entry |  |  |
| DRB-Identity[1] | DRB-Identity using condition DRBj | Entry 1 j is the ID of the DRB established during the preamble which is allocated according to internal TTCN mapping |  |
| } |  |  |  |
| sdt-MAC-PHY-CG-Config-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cg-SDT-ConfigInitialBWP-NUL-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| pusch-Config-r17 CHOICE { |  |  |  |
| setup | PUSCH-Config | TS 38.508-1[4] Table 4.6.3-118: PUSCH-Config |  |
| } |  |  |  |
| configuredGrantConfigToAddModList-r17 SEQUENCE (SIZE (1..maxNrofConfiguredGrantConfig-r16)) OF ConfiguredGrantConfig { |  |  |  |
| ConfiguredGrantConfig[1] | ConfiguredGrantConfig | Table 7.1.1.13.8.3.3-8 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cg-SDT-ConfigInitialBWP-SUL-r17 | Not present |  |  |
| cg-SDT-ConfigInitialBWP-DL-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| pdcch-Config-r17 | Not present |  |  |
| pdsch-Config-r17 CHOICE { |  |  |  |
| setup | PDSCH-Config | TS 38.508-1[4] Table 4.6.3-100: PDSCH-Config |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cg-SDT-TimeAlignmentTimer-r17 | ms10240 | TimeAlignmentTimer(10.24s) |  |
| cg-SDT-RSRP-ThresholdSSB-r17 | 60 | (IE value – 156) dBm = - 96 dBm |  |
| cg-SDT-TA-ValidationConfig-r17 | Not present | UE does not perform RSRP based TA validation |  |
| } |  |  |  |
| cg-SDT-CS-RNTI-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.13.8.3.3-9: ConfiguredGrantConfig (Table 7.1.1.13.8.3.3-8)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-26, condition CG_Config_Type1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ConfiguredGrantConfig ::= SEQUENCE { |  |  |  |
| rrc-ConfiguredUplinkGrant SEQUENCE{ |  |  |  |
| frequencyDomainAllocation | BIT STRING (SIZE(18) | Equals to NBWPsize * (NBWPsize – LRB + 1) + (NBWPsize -1 - RBstart), where LRB = 24 PRB, RBstart = 0, NBWPsize is the size [PRBs] of the carrier bandwidth and contained in TS.38.508-1 [4] clause 6.2.3.1 | SCS15 AND CBW5, SCS30 AND CBW10 |
|  | BIT STRING (SIZE(18) | Equal to NBWPsize * (LRB-1) + RBstart), where LRB = 24 PRB, RBstart = 0, NBWPsize is the size [PRBs] of the carrier bandwidth and contained in TS.38.508-1 [4] clause 6.2.3.1 |  |
| dmrs-SeqInitialization | 0 |  |  |
| mcsAndTBS | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| Note: Configured UL grant for CG-SDT is 888 bits (LRBs & IMCS as per 38.523-3[3] annex B) is chosen to enable UE to transmit RRCResumeRequest and Loop backed RLC PDU in a MAC PDU ( 97 bytes for Loop backed RLC PDU + 2 bytes for MAC Sub Header + 6 bytes for RRCResumeRequest message + 2 bytes for MAC Sub Header + 4 bytes for BSR and Padding ) |  |  |  |

Table 7.1.1.13.8.3.3-10: RRCResumeRequest (steps 3, 14 and 27, Table 7.1.1.13.8.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCResumeRequest ::= SEQUENCE { |  |  |  |
| rrcResumeRequest SEQUENCE { |  |  |  |
| resumeCause | mt-SDT-v1810 |  |  |
| } |  |  |  |
| } |  |  |  |

#### 7.1.1.14	Network energy saving

##### 7.1.1.14.1	Cell DTX/DRX adaptation

###### 7.1.1.14.1.1	Cell DTX operation / Correct handling of DL assignment / (De)activation by RRC configuration

7.1.1.14.1.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { cell DTX is configured and activated by RRC configuration and a new DL transmission is indicated on the PDCCH during the cell DTX Active Period }

then { UE monitors the PDCCH during the cell DTX Active Period PDCCH occasions }

}

(2)

with { UE in RRC_CONNECTED state with cell DTX is configured and activated }

ensure that {

when { cell DTX is deactivated by RRC configuration and a new DL transmission is indicated on the PDCCH outside the cell DTX Active Period }

then { UE starts monitors the PDCCH outside the cell DTX Active Period PDCCH occasions }

}

7.1.1.14.1.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.321, clause 5.34.1 and clause 5.34.2. Unless otherwise stated these are Rel-18 requirements.

[TS 38.321, clause 5.34.1]

Each Serving Cell may be configured by RRC with a periodic cell DTX pattern (i.e., Active and Non-Active Periods). The cell DTX operation controls UE's monitoring activity of PDCCH and configured downlink assignments in RRC_CONNECTED. For all activated Serving Cells with cell DTX configured and activated, the MAC entity may monitor PDCCH and configured downlink assignments using the cell DTX operation specified in clause 5.34.2.

Each Serving Cell may be configured by RRC with a periodic cell DRX pattern (i.e., Active and Non-Active Periods). The cell DRX operation controls Scheduling Request and configured uplink grant transmission activity in RRC_CONNECTED. For all activated Serving Cells with cell DRX configured and activated, the MAC entity may transmit configured uplink grant transmissions and Scheduling Request using the cell DRX operation specified in clause 5.34.3.

RRC controls cell DTX and cell DRX operation by configuring the following parameters in cellDTX-DRX-Config per Serving Cell:

-	cellDTX-DRX-ConfigType: defines whether only cell DTX is configured, only cell DRX is configured, or both are configured;

-	cellDTX-DRX-onDurationTimer: the active duration at the beginning of a cell DTX/DRX cycle;

-	cellDTX-DRX-StartOffset: defines the subframe where the cell DTX/DRX cycle starts;

-	cellDTX-DRX-SlotOffset: the delay before starting the cellDTX-DRX-onDurationTimer;

-	cellDTX-DRX-Cycle: the cell DTX/DRX cycle period.

-	ActivationStatus: the initial activation status of cell DTX and cell DRX operation.

[TS 38.321, clause 5.34.2]

Cell DTX is configured if cellDTX-DRX-ConfigType is set to dtx or dtxdrx. Cell DTX operation is activated and deactivated for each Serving Cell by:

-	receiving a cell DTX indication from lower layers indicating activation or deactivation of cell DTX operation, as specified in TS 38.213 [6];

-	configuring cellDTX-DRX-Config by upper layers: if cell DTX is configured and cellDTX-DRX-ActivationStatus is set to activated, cell DTX operation is activated upon cell DTX configuration; if cell DTX is configured and cellDTX-DRX-ActivationStatus is set to deactivated, cell DTX operation is deactivated upon cell DTX configuration; if cellDTX-DRX-Config is released, cell DTX operation is deactivated and all the corresponding configurations are released.

When cell DTX is configured and activated for a Serving Cell, the cell DTX Active Period includes the time while:

-	cellDTX-DRX-onDurationTimer is running for the associated Serving Cell.

For each Serving Cell configured with cell DTX, the MAC entity shall:

1>	if cell DTX is activated for this Serving Cell:

2>	if [(SFN × 10) + subframe number] modulo (cellDTX-DRX-Cycle) = (cellDTX-DRX-StartOffset):

3>	start cellDTX-DRX-onDurationTimer for this serving cell after cellDTX-DRX-SlotOffset from the beginning of the subframe.

1>	if cell DTX operation is deactivated for this Serving Cell; or

1>	if the Serving Cell is in the cell DTX Active Period:

2>	monitor PDCCH on this Serving Cell, as specified in TS 38.213 [6] and other clauses of this specification.

1>	if any drx-RetransmissionTimerDL, drx-RetransmissionTimerUL or drx-RetransmissionTimerSL (as described in clause 5.7) is running on any Serving Cell in the DRX group of this Serving Cell; or

1>	if ra-ContentionResolutionTimer (as described in clause 5.1.5) or msgB-ResponseWindow (as described in clause 5.1.4a) is running; or

1>	if a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4 or 5.22.1.5); or

1>	if a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clauses 5.1.4 and 5.1.4a):

2>	monitor PDCCH on the Serving Cells in the DRX group of this Serving Cell, as specified in TS 38.213 [6] and other clauses of this specification.

1>	if ra-ResponseWindow (as described in clause 5.1.4) is running and this Serving Cell is the SpCell:

2>	monitor PDCCH on this Serving Cell (as described in clause 5.1.4).

For each Serving Cell configured with cell DTX, the MAC entity need not:

1>	if cell DTX operation is activated and the Serving Cell is not in the cell DTX Active Period:

2>	monitor PDCCH for the MAC entity's RNTIs listed in clauses 5.7 and 5.7b, irrespective of the requirements of clauses 5.7 and 5.7b, unless stated otherwise in this clause;

2>	instruct the physical layer to receive transport block on the DL-SCH of this Serving Cell according to a configured downlink assignment for SPS;

2>	indicate the presence of a configured downlink assignment and deliver the stored HARQ information to the HARQ entity;

2>	set the HARQ Process ID to the HARQ Process ID associated with the PDSCH duration of a configured downlink assignment;

2>	consider the NDI bit for the HARQ process corresponding to the PDSCH duration of a configured downlink assignment to have been toggled for the configured downlink assignment.

7.1.1.14.1.1.3	Test description

7.1.1.14.1.1.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that set to return no data in uplink.

7.1.1.14.1.1.3.2	Test procedure sequence

Table 7.1.1.14.1.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits an RRCReconfiguration message to configure and activate Cell DTX pattern. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | In the first PDCCH occasion when the cellDTX-DRX-onDurationTimer is running, the SS indicates the transmission of a DL MAC PDU on the PDCCH. | <-- | MAC PDU | - | - |
| 4 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 3 within 10s? | --> | HARQ ACK | 1 | P |
| 5 | SS transmits an RRCReconfiguration message to deactivate Cell DTX. | <-- | NR RRC: RRCReconfiguration |  |  |
| 6 | The UE transmits RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete |  |  |
| 7 | In the first PDCCH occasion after the cellDTX-DRX-onDurationTimer expired, the SS indicates the transmission of a DL MAC PDU on the PDCCH. | <-- | MAC PDU | - | - |
| 8 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 7? | --> | HARQ ACK | 2 | P |

7.1.1.14.1.1.3.3	Specific message contents

Table 7.1.1.14.1.1.3.3-1: RRCReconfiguration (step 1 and step 5, Table 7.1.1.14.1.1.3.2-1)

| Derivation Path: TS 38.508-1 [6], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| secondaryCellGroup |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig | As defined in Table 7.1.1.14.1.1.3.3-2 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.14.1.1.3.3-2: CellGroupConfig (Table 7.1.1.14.1.1.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |  |
| mac-CellGroupConfig SEQUENCE { |  |  |  |  |
| drx-Config CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| drx-onDurationTimer |  | ms40 |  |  |
| drx-InactivityTimer |  | ms10 |  |  |
| drx-HARQ-RTT-TimerDL |  | 56 | Number of slots=4 due to number of symbol per slot=14 | =0,1,2,3,4 ( 2 with normal CP) |
|  |  | 48 | Number of slots=4 due to number of symbol per slot=12 | = 2 with external CP |
| drx-HARQ-RTT-TimerUL |  | 56 | Number of slots=4 due to number of symbol per slot=14 | =0,1,2,3,4 ( 2 with normal CP) |
|  |  | 48 | Number of slots=4 due to number of symbol per slot=12 | = 2 with external CP |
| drx-RetransmissionTimerDL |  | sl8 |  |  |
| drx-RetransmissionTimerUL |  | sl8 |  |  |
| drx-LongCycleStartOffset CHOICE { |  |  |  |  |
| ms640 |  | 7 |  |  |
| } |  |  |  |  |
| shortDRX |  | Not present |  |  |
| drx-SlotOffset |  | ms0 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |  |
| servingCellConfig |  | ServingCellConfig | Table 7.1.1.14.1.1.3.3-3 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.14.1.1.3.3-3: ServingCellConfig (Table 7.1.1.14.1.1.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| ServingCellConfig SEQUENCE { |  |  |  |  |
| cellDTX-DRX-Config-r18 CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| cellDTX-DRX-onDurationTimer-r18 CHOICE { |  |  |  |  |
| milliSeconds |  | ms5 |  |  |
| } |  |  |  |  |
| cellDTX-DRX-CycleStartOffset-r18 CHOICE { |  |  |  |  |
| ms640 |  | 0 |  |  |
| } |  |  |  |  |
| cellDTX-DRX-SlotOffset-r18 |  | 7 |  |  |
| cellDTX-DRX-ConfigType-r18 |  | dtx |  |  |
| cellDTX-DRX-ActivationStatus-r18 |  | activated |  | Step 1 |
|  |  | deactivated |  | Step 5 |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.14.1.1.3.3-4: Void

Table 7.1.1.14.1.1.3.3-5: Void

Table 7.1.1.14.1.1.3.3-6: Void

###### 7.1.1.14.1.2	Cell DTX operation / Correct handling of DL assignment / (De)activation by DCI format 2_9

7.1.1.14.1.2.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and cell DTX is configured and deactivated by RRC configuration}

ensure that {

when { UE receives DCI format 2_9 activating cell DTX and a new DL transmission is indicated on the PDCCH during the cell DTX Active Period }

then { UE monitors the PDCCH during the cell DTX Active Period PDCCH occasions }

}

(2)

with { UE in RRC_CONNECTED state with cell DTX is configured and activated }

ensure that {

when { UE receives DCI format 2_9 deactivating cell DTX and a new DL transmission is indicated on the PDCCH outside the cell DTX Active Period }

then { UE starts monitors the PDCCH outside the cell DTX Active Period PDCCH occasions }

}

7.1.1.14.1.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.212, clause 7.3.1.3.10, TS 38.213, clause 11.5, TS 38.321, clause 5.34.1 and clause 5.34.2. Unless otherwise stated these are Rel-18 requirements.

[TS 38.212, clause 7.3.1.3.10]

DCI format 2_9 is used for activating or de-activating the cell DTX and/or DRX configuration of one or multiple serving cells for one or more UEs, and/or for providing NES-mode indication of the primary cell for one or more UEs.

The following information is transmitted by means of the DCI format 2_9 with CRC scrambled by cellDTRX-RNTI:

-	block number 1, block number 2,…, block number N

where the starting position of a block associated with a serving cell is determined by the parameter positionInDCI-cellDTRX provided by higher layers for the UE.

If the UE is configured to monitor DCI 2_9 with CRC scrambled by cellDTRX-RNTI, one or more blocks are configured for the UE by higher layers, with the following fields defined for each block:

-	Cell DTX/DRX indication - number of bits determined by the following:

-	If higher layer parameter cellDTXDRX-L1activation is configured

-	2 bits as defined in Clause 11.5 of [5, TS38.213] if cellDTXDRXconfigType is configured to dtxdrx for the associated serving cell of the block, with the MSB corresponding to cell DTX configuration and the LSB corresponding to cell DRX configuration;

-	1 bit as defined in Clause 11.5 of [5, TS38.213] if cellDTXDRXconfigType is configured to either dtx or drx for the associated serving cell of the block;

-	0 bit otherwise.

-	NES-mode indication – 1 bit indicating NES-specific CHO execution condition as defined in Clause 11.5 of [5, TS38.213], if the higher layer parameter nesEvent is configured and the associated serving cell of the block is primary cell; 0 bit otherwise.

The size of DCI format 2_9 is indicated by the higher layer parameter sizeDCI-2-9.

[TS 38.213, clause 11.5]

A UE configured for operation on a serving cell according to one or both of a cell DTX operation and a cell DRX operation by cellDTXDRX-Config for the serving cell [11, TS 38.321], can be additionally provided by dci-Format2-9 a Type3-PDCCH CSS set to monitor PDCCH for detection of DCI format 2_9 as described in clause 10.1 during Active Time [11, TS 38.321], and a location in DCI format 2_9 by positionInDCI-cellDTRX of a cell DTX/DRX indication field for the serving cell and/or a NES-mode indication field for the PCell

-	if the UE is configured with both cell DTX operation and cell DRX operation for the serving cell and if cellDTXDRX-L1activation is provided, the cell DTX/DRX indication field includes two bits where the first bit indicates the cell DTX operation and the second bit indicates the cell DRX operation

-	if the UE is configured with only one of the cell DTX operation and cell DRX operation for the serving cell and if cellDTXDRX-L1activation is provided, the cell DTX/DRX indication field includes one bit indicating one of the cell DTX operation and cell DRX operation, respectively, for the serving cell

-	a '0' value for a bit of the cell DTX/DRX indication field indicates deactivation of cell DTX or of cell DRX

-	a '1' value for a bit of the cell DTX/DRX indication field indicates activation of cell DTX or of cell DRX

-	if the serving cell is configured with a SUL carrier, the cell DTX/DRX indication field indication for activation or deactivation of cell DRX applies to both the UL carrier and the SUL carrier

-	if nesEvent is configured, the NES-mode indication field includes one bit indicating NES-specific CHO execution condition, as described in [12, TS 38.331]

-	a '0' value for the NES-mode indication field indicates NES-specific CHO execution condition is disabled

-	a '1' value for the NES-mode indication field, indicates NES-specific CHO execution condition is enabled

A UE does not expect to monitor PDCCH for detection of DCI format 2_9 on more than one serving cells of one cell group.

When a UE receives in slot m on the active DL BWP of a first serving cell a PDCCH providing DCI format 2_9 that indicates a change in activation or deactivation of a current cell DTX operation or cell DRX operation for a second serving cell, the UE operates on the second serving cell according to the indicated cell DTX operation or cell DRX operation starting from a slot on the active DL BWP or on the active UL BWP of the second serving cell, respectively, that is not before the beginning of the slot m+d on the active DL BWP of the first serving cell where d is a number of slots for the SCS of the active DL BWP of the first serving cell in Table 11.5-1.

Table 11.5-1: Minimum time gap value d

| SCS (kHz) | Number of slots |
| --- | --- |
| 15 | 3 |
| 30 | 6 |
| 60 | 12 |
| 120 | 24 |
| 480 | 96 |
| 960 | 192 |

[TS 38.321, clause 5.34.1]

Each Serving Cell may be configured by RRC with a periodic cell DTX pattern (i.e., Active and Non-Active Periods). The cell DTX operation controls UE's monitoring activity of PDCCH and configured downlink assignments in RRC_CONNECTED. For all activated Serving Cells with cell DTX configured and activated, the MAC entity may monitor PDCCH and configured downlink assignments using the cell DTX operation specified in clause 5.34.2.

Each Serving Cell may be configured by RRC with a periodic cell DRX pattern (i.e., Active and Non-Active Periods). The cell DRX operation controls Scheduling Request and configured uplink grant transmission activity in RRC_CONNECTED. For all activated Serving Cells with cell DRX configured and activated, the MAC entity may transmit configured uplink grant transmissions and Scheduling Request using the cell DRX operation specified in clause 5.34.3.

RRC controls cell DTX and cell DRX operation by configuring the following parameters in cellDTX-DRX-Config per Serving Cell:

-	cellDTX-DRX-ConfigType: defines whether only cell DTX is configured, only cell DRX is configured, or both are configured;

-	cellDTX-DRX-onDurationTimer: the active duration at the beginning of a cell DTX/DRX cycle;

-	cellDTX-DRX-StartOffset: defines the subframe where the cell DTX/DRX cycle starts;

-	cellDTX-DRX-SlotOffset: the delay before starting the cellDTX-DRX-onDurationTimer;

-	cellDTX-DRX-Cycle: the cell DTX/DRX cycle period.

-	ActivationStatus: the initial activation status of cell DTX and cell DRX operation.

[TS 38.321, clause 5.34.2]

Cell DTX is configured if cellDTX-DRX-ConfigType is set to dtx or dtxdrx. Cell DTX operation is activated and deactivated for each Serving Cell by:

-	receiving a cell DTX indication from lower layers indicating activation or deactivation of cell DTX operation, as specified in TS 38.213 [6];

-	configuring cellDTX-DRX-Config by upper layers: if cell DTX is configured and cellDTX-DRX-ActivationStatus is set to activated, cell DTX operation is activated upon cell DTX configuration; if cell DTX is configured and cellDTX-DRX-ActivationStatus is set to deactivated, cell DTX operation is deactivated upon cell DTX configuration; if cellDTX-DRX-Config is released, cell DTX operation is deactivated and all the corresponding configurations are released.

When cell DTX is configured and activated for a Serving Cell, the cell DTX Active Period includes the time while:

-	cellDTX-DRX-onDurationTimer is running for the associated Serving Cell.

For each Serving Cell configured with cell DTX, the MAC entity shall:

1>	if cell DTX is activated for this Serving Cell:

2>	if [(SFN × 10) + subframe number] modulo (cellDTX-DRX-Cycle) = (cellDTX-DRX-StartOffset):

3>	start cellDTX-DRX-onDurationTimer for this serving cell after cellDTX-DRX-SlotOffset from the beginning of the subframe.

1>	if cell DTX operation is deactivated for this Serving Cell; or

1>	if the Serving Cell is in the cell DTX Active Period:

2>	monitor PDCCH on this Serving Cell, as specified in TS 38.213 [6] and other clauses of this specification.

1>	if any drx-RetransmissionTimerDL, drx-RetransmissionTimerUL or drx-RetransmissionTimerSL (as described in clause 5.7) is running on any Serving Cell in the DRX group of this Serving Cell; or

1>	if ra-ContentionResolutionTimer (as described in clause 5.1.5) or msgB-ResponseWindow (as described in clause 5.1.4a) is running; or

1>	if a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4 or 5.22.1.5); or

1>	if a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clauses 5.1.4 and 5.1.4a):

2>	monitor PDCCH on the Serving Cells in the DRX group of this Serving Cell, as specified in TS 38.213 [6] and other clauses of this specification.

1>	if ra-ResponseWindow (as described in clause 5.1.4) is running and this Serving Cell is the SpCell:

2>	monitor PDCCH on this Serving Cell (as described in clause 5.1.4).

For each Serving Cell configured with cell DTX, the MAC entity need not:

1>	if cell DTX operation is activated and the Serving Cell is not in the cell DTX Active Period:

2>	monitor PDCCH for the MAC entity's RNTIs listed in clauses 5.7 and 5.7b, irrespective of the requirements of clauses 5.7 and 5.7b, unless stated otherwise in this clause;

2>	instruct the physical layer to receive transport block on the DL-SCH of this Serving Cell according to a configured downlink assignment for SPS;

2>	indicate the presence of a configured downlink assignment and deliver the stored HARQ information to the HARQ entity;

2>	set the HARQ Process ID to the HARQ Process ID associated with the PDSCH duration of a configured downlink assignment;

2>	consider the NDI bit for the HARQ process corresponding to the PDSCH duration of a configured downlink assignment to have been toggled for the configured downlink assignment.

7.1.1.14.1.2.3	Test description

7.1.1.14.1.2.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that set to return no data in uplink.

7.1.1.14.1.2.3.2	Test procedure sequence

Table 7.1.1.14.1.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits an RRCReconfiguration message to configure Cell DTX pattern. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | The SS transmits DCI format 2_9 indicating to activate cell DTX. (Note 1) | <-- | PDCCH (DCI 2-9) | - | - |
| 4 | Wait 5ms to ensure DCI format 2_9 indication take effects. | - | - | - | - |
| 5 | In the first PDCCH occasion when the cellDTX-DRX-onDurationTimer is running, the SS indicates the transmission of a DL MAC PDU on the PDCCH. | <-- | MAC PDU | - | - |
| 6 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 5 within 10s? | --> | HARQ ACK | 1 | P |
| 7 | The SS transmits DCI format 2_9 before the start of next cellDTX-DRX-onDurationTimer and the DCI format 2_9 indicates to deactivate cell DTX. (Note 2) | <-- | PDCCH (DCI 2-9) | - | - |
| 8 | Wait 5ms to ensure DCI format 2_9 indication take effects. | - | - | - | - |
| 9 | In the first PDCCH occasion after the cellDTX-DRX-onDurationTimer expired, the SS indicates the transmission of a DL MAC PDU on the PDCCH. | <-- | MAC PDU | - | - |
| 10 | Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 9? | --> | HARQ ACK | 2 | P |
| Note 1: The cell DTX/DRX indication field is value '1' in the DCI 2-9. Note 2: The cell DTX/DRX indication field is value '0' in the DCI 2-9. |  |  |  |  |  |

7.1.1.14.1.2.3.3	Specific message contents

Table 7.1.1.14.1.2.3.3-1: RRCReconfiguration (step 1, Table 7.1.1.14.1.2.3.2-1)

| Derivation Path: TS 38.508-1 [6], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| secondaryCellGroup |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.14.1.2.3.3-2: CellGroupConfig (Table 7.1.1.14.1.2.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |  |
| mac-CellGroupConfig SEQUENCE { |  |  |  |  |
| drx-Config CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| drx-onDurationTimer |  | ms40 |  |  |
| drx-InactivityTimer |  | ms10 |  |  |
| drx-HARQ-RTT-TimerDL |  | 56 | Number of slots=4 due to number of symbol per slot=14 | =0,1,2,3,4 ( 2 with normal CP) |
|  |  | 48 | Number of slots=4 due to number of symbol per slot=12 | = 2 with external CP |
| drx-HARQ-RTT-TimerUL |  | 56 | Number of slots=4 due to number of symbol per slot=14 | =0,1,2,3,4 ( 2 with normal CP) |
|  |  | 48 | Number of slots=4 due to number of symbol per slot=12 | = 2 with external CP |
| drx-RetransmissionTimerDL |  | sl8 |  |  |
| drx-RetransmissionTimerUL |  | sl8 |  |  |
| drx-LongCycleStartOffset CHOICE { |  |  |  |  |
| ms640 |  | 7 |  |  |
| } |  |  |  |  |
| shortDRX |  | Not present |  |  |
| drx-SlotOffset |  | ms0 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| physicalCellGroupConfig |  | PhysicalCellGroupConfig | Table 7.1.1.14.1.2.3.3-3 |  |
| spCellConfig SEQUENCE { |  |  |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |  |
| servingCellConfig |  | ServingCellConfig | Table 7.1.1.14.1.2.3.3-4 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.14.1.2.3.3-3: PhysicalCellGroupConfig (Table 7.1.1.14.1.2.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PhysicalCellGroupConfig::= SEQUENCE { |  |  |  |  |
| cellDTRX-DCI-config-r18 CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| cellDTRX-RNTI-r18 |  | RNTI-Value |  |  |
| sizeDCI-2-9-r18 |  | 1 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.14.1.2.3.3-4: ServingCellConfig (Table 7.1.1.14.1.2.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| ServingCellConfig SEQUENCE { |  |  |  |  |
| initialDownlinkBWP ::= SEQUENCE { |  |  |  |  |
| pdcch-Config CHOICE { |  |  |  |  |
| setup |  | PDCCH-Config | Table 7.1.1.14.1.2.3.3-5 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| cellDTX-DRX-Config-r18 CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| cellDTX-DRX-onDurationTimer-r18 CHOICE { |  |  |  |  |
| milliSeconds |  | ms5 |  |  |
| } |  |  |  |  |
| cellDTX-DRX-CycleStartOffset-r18 CHOICE { |  |  |  |  |
| ms640 |  | 7 |  |  |
| } |  |  |  |  |
| cellDTX-DRX-SlotOffset-r18 |  | 0 |  |  |
| cellDTX-DRX-ConfigType-r18 |  | dtx |  |  |
| cellDTX-DRX-ActivationStatus-r18 |  | deactivated |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| positionInDCI-cellDTRX-r18 |  | 0 |  |  |
| cellDTX-DRX-L1activation-r18 |  | enabled |  |  |
| } |  |  |  |  |

Table 7.1.1.14.1.2.3.3-5: PDCCH-Config (Table 7.1.1.14.1.2.3.3-4)

Derivation Path: TS 38.508-1 [4], Table 4.6.3-95 with condition DCI_2_9

###### 7.1.1.14.1.3	Cell DRX operation / Correct handling of UL grant / (De)activation by RRC configuration

7.1.1.14.1.3.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with SR resource on PUCCH is configured }

ensure that {

when { cell DRX is configured and activated by RRC signalling and UE has UL data available for transmission and UE has no UL-SCH resources available and SR_COUNTER is less than sr-TransMax }

then { UE starts the cellDTX-DRX-onDurationTimer and does not transmits a SR outside the cell DRX Active Period }

}

(2)

with { UE in RRC_CONNECTED state with cell DRX is configured and activated }

ensure that {

when { cell DTX is deactivated by RRC configuration and UE has UL data available for transmission and UE has no UL-SCH resources available and SR_COUNTER is less than sr-TransMax }

then { UE transmits a SR outside the cell DRX Active Period }

}

7.1.1.14.1.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.321, clause 5.34.1 and clause 5.34.3. Unless otherwise stated these are Rel-18 requirements.

[TS 38.321, clause 5.34.1]

Each Serving Cell may be configured by RRC with a periodic cell DTX pattern (i.e., Active and Non-Active Periods). The cell DTX operation controls UE's monitoring activity of PDCCH and configured downlink assignments in RRC_CONNECTED. For all activated Serving Cells with cell DTX configured and activated, the MAC entity may monitor PDCCH and configured downlink assignments using the cell DTX operation specified in clause 5.34.2.

Each Serving Cell may be configured by RRC with a periodic cell DRX pattern (i.e., Active and Non-Active Periods). The cell DRX operation controls Scheduling Request and configured uplink grant transmission activity in RRC_CONNECTED. For all activated Serving Cells with cell DRX configured and activated, the MAC entity may transmit configured uplink grant transmissions and Scheduling Request using the cell DRX operation specified in clause 5.34.3.

RRC controls cell DTX and cell DRX operation by configuring the following parameters in cellDTX-DRX-Config per Serving Cell:

-	cellDTX-DRX-ConfigType: defines whether only cell DTX is configured, only cell DRX is configured, or both are configured;

-	cellDTX-DRX-onDurationTimer: the active duration at the beginning of a cell DTX/DRX cycle;

-	cellDTX-DRX-StartOffset: defines the subframe where the cell DTX/DRX cycle starts;

-	cellDTX-DRX-SlotOffset: the delay before starting the cellDTX-DRX-onDurationTimer;

-	cellDTX-DRX-Cycle: the cell DTX/DRX cycle period.

-	ActivationStatus: the initial activation status of cell DTX and cell DRX operation.

[TS 38.321, clause 5.34.3]

Cell DRX is configured if cellDTX-DRX-ConfigType is set to drx or dtxdrx. Cell DRX operation is activated and deactivated for each Serving Cell by:

-	receiving a cell DRX indication from lower layers indicating activation or deactivation of cell DRX operation, as specified in TS 38.213 [6];

-	configuring cellDTX-DRX-Config by upper layers: if cell DRX is configured and ActivationStatus is set to activated, cell DRX operation is activated upon cell DRX configuration; if cell DRX is configured and -ActivationStatus is set to deactivated, cell DRX operation is deactivated upon cell DRX configuration; if cellDTX-DRX-Config is released, cell DRX operation is deactivated and all the corresponding configurations are released.

When cell DRX is configured and activated for a Serving Cell, the cell DRX Active Period includes the time while:

-	cellDTX-DRX-onDurationTimer is running for the associated Serving Cell.

For each Serving Cell configured with cell DRX, the MAC entity shall:

1>	if cell DRX is activated for this Serving Cell:

2>	if [(SFN × 10) + subframe number] modulo (cellDTX-DRX-Cycle) = (cellDTX-DRX-StartOffset):

3>	start cellDTX-DRX-onDurationTimer for this serving cell after cellDTX-DRX-SlotOffset from the beginning of the subframe.

1>	if cell DRX is activated and the Serving Cell is not in the cell DRX Active Period:

2>	not instruct the physical layer to signal a SR on a PUCCH resource for SR;

2>	not increment the SR_COUNTER for a SR;

2>	not start the sr-ProhibitTimer for a SR;

2>	not deliver any configured uplink grant and the associated HARQ information to the HARQ entity;

2>	not instruct a HARQ process associated with a configured uplink grant to trigger a new transmission or a retransmission;

2>	not report CSI on PUCCH and semi-persistent CSI configured on PUSCH;

2>	if an emergency service is initiated by upper layers and this Serving Cell is the SpCell:

3>	initiate a Random Access procedure (as specified in clause 5.1.1).

NOTE:	How the MAC layer in the UE is aware of an ongoing emergency service is up to UE implementation.

7.1.1.14.1.3.3	Test description

7.1.1.14.1.3.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0.

7.1.1.14.1.3.3.2	Test procedure sequence

Table 7.1.1.14.1.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits an RRCReconfiguration message to configure and activate Cell DRX pattern. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | Wait 5ms to ensure Cell DRX is activated. | - | - | - | - |
| 3A | The SS transmits a MAC PDU. | <-- | MAC PDU | - | - |
| 4 | Check: Does the UE repeatedly transmit a Scheduling Request on the all available PUCCH when the Serving Cell is not in the cell DRX Active Period within 3s? | --> | SR | 1 | F |
| 4A | The SS transmits an UL grant to allocate UL-SCH resources that are enough to transmit looped back PDU | <-- | (UL Grant) | - | - |
| 4B | Does UE transmit a MAC PDU? | --> | MAC PDU | - | - |
| 5 | SS transmits an RRCReconfiguration message to deactivate Cell DRX. | <-- | NR RRC: RRCReconfiguration |  |  |
| 6 | The UE transmits RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 7 | Wait 5ms to ensure Cell DRX is deactivated. | - | - | - | - |
| 7A | The SS transmits a MAC PDU. | <-- | MAC PDU | - | - |
| 8 | Check: Does the UE repeatedly transmit a Scheduling Request on the any available PUCCH when the Serving Cell is not in the cell DRX Active Period within 100ms? Note: The default period for SR is 10ms and default sr-TransMax is 16. 100ms < 16*10ms, so the SR_COUNTER is less than sr-TransMax. | --> | SR | 2 | P |
| 9 | The SS transmits an UL grant to allocate UL-SCH resources that are enough to transmit looped back PDU | <-- | (UL Grant ) | - | - |
| 10 | Check: Does the UE transmit a MAC PDU? | --> | MAC PDU | - | - |

7.1.1.14.1.3.3.3	Specific message contents

Table 7.1.1.14.1.3.3.3-1: RRCReconfiguration (step 1, Table 7.1.1.14.1.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| secondaryCellGroup |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig | Table 7.1.1.14.1.3.3.3-2 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.14.1.3.3.3-2: CellGroupConfig (Table 7.1.1.14.1.3.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |  |
| servingCellConfig |  | ServingCellConfig | Table 7.1.1.14.1.3.3.3-3 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.14.1.3.3.3-3: ServingCellConfig (Table 7.1.1.14.1.3.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| ServingCellConfig SEQUENCE { |  |  |  |  |
| cellDTX-DRX-Config-r18 CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| cellDTX-DRX-onDurationTimer-r18 CHOICE { |  |  |  |  |
| milliSeconds |  | ms1 |  |  |
| } |  |  |  |  |
| cellDTX-DRX-CycleStartOffset-r18 CHOICE { |  |  |  |  |
| ms640 |  | 0 |  |  |
| } |  |  |  |  |
| cellDTX-DRX-SlotOffset-r18 |  | 0 |  |  |
| cellDTX-DRX-ConfigType-r18 |  | drx |  |  |
| cellDTX-DRX-ActivationStatus-r18 |  | activated |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.14.1.3.3.3-4: RRCReconfiguration (step 6, Table 7.1.1.14.1.3.3.2-1)

| Derivation Path: TS 38.508-1 [6], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| secondaryCellGroup |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig | As defined in Table 7.1.1.14.1.3.3.3-5 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.14.1.3.3.3-5: CellGroupConfig (Table 7.1.1.14.1.3.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |  |
| servingCellConfig |  | ServingCellConfig | Table 7.1.1.14.1.3.3.3-6 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.14.1.3.3.3-6: ServingCellConfig (Table 7.1.1.14.1.3.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| ServingCellConfig SEQUENCE { |  |  |  |  |
| cellDTX-DRX-Config-r18 CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| cellDTX-DRX-onDurationTimer-r18 CHOICE { |  |  |  |  |
| milliSeconds |  | ms1 |  |  |
| } |  |  |  |  |
| cellDTX-DRX-CycleStartOffset-r18 CHOICE { |  |  |  |  |
| ms640 |  | 0 |  |  |
| } |  |  |  |  |
| cellDTX-DRX-SlotOffset-r18 |  | 0 |  |  |
| cellDTX-DRX-ConfigType-r18 |  | drx |  |  |
| cellDTX-DRX-ActivationStatus-r18 |  | deactivated |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

###### 7.1.1.14.1.4	Cell DRX operation / Correct handling of UL grant / (De)activation by DCI format 2_9

7.1.1.14.1.4.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with SR resource on PUCCH is configured and cell DRX is configured and deactivated by RRC configuration }

ensure that {

when { UE receives DCI format 2_9 activating cell DRX and UE has UL data available for transmission and UE has no UL-SCH resources available and SR_COUNTER is less than sr-TransMax }

then { UE starts the cellDTX-DRX-onDurationTimer and does not transmit a SR outside the cell DRX Active Period }

}

(2)

with { UE in RRC_CONNECTED state with cell DRX is configured and activated }

ensure that {

when { UE receives DCI format 2_9 deactivating cell DRX and UE has UL data available for transmission and UE has no UL-SCH resources available and SR_COUNTER is less than sr-TransMax }

then { UE transmits a SR outside the cell DRX Active Period }

}

7.1.1.14.1.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.212, clause 7.3.1.3.10, TS 38.213, clause 11.5, TS 38.321, clause 5.34.1 and clause 5.34.3. Unless otherwise stated these are Rel-18 requirements.

[TS 38.212, clause 7.3.1.3.10]

DCI format 2_9 is used for activating or de-activating the cell DTX and/or DRX configuration of one or multiple serving cells for one or more UEs, and/or for providing NES-mode indication of the primary cell for one or more UEs.

The following information is transmitted by means of the DCI format 2_9 with CRC scrambled by cellDTRX-RNTI:

-	block number 1, block number 2,…, block number N

where the starting position of a block associated with a serving cell is determined by the parameter positionInDCI-cellDTRX provided by higher layers for the UE.

If the UE is configured to monitor DCI 2_9 with CRC scrambled by cellDTRX-RNTI, one or more blocks are configured for the UE by higher layers, with the following fields defined for each block:

-	Cell DTX/DRX indication - number of bits determined by the following:

-	If higher layer parameter cellDTXDRX-L1activation is configured

-	2 bits as defined in Clause 11.5 of [5, TS38.213] if cellDTXDRXconfigType is configured to dtxdrx for the associated serving cell of the block, with the MSB corresponding to cell DTX configuration and the LSB corresponding to cell DRX configuration;

-	1 bit as defined in Clause 11.5 of [5, TS38.213] if cellDTXDRXconfigType is configured to either dtx or drx for the associated serving cell of the block;

-	0 bit otherwise.

-	NES-mode indication – 1 bit indicating NES-specific CHO execution condition as defined in Clause 11.5 of [5, TS38.213], if the higher layer parameter nesEvent is configured and the associated serving cell of the block is primary cell; 0 bit otherwise.

The size of DCI format 2_9 is indicated by the higher layer parameter sizeDCI-2-9.

[TS 38.213, clause 11.5]

A UE configured for operation on a serving cell according to one or both of a cell DTX operation and a cell DRX operation by cellDTXDRX-Config for the serving cell [11, TS 38.321], can be additionally provided by dci-Format2-9 a Type3-PDCCH CSS set to monitor PDCCH for detection of DCI format 2_9 as described in clause 10.1 during Active Time [11, TS 38.321], and a location in DCI format 2_9 by positionInDCI-cellDTRX of a cell DTX/DRX indication field for the serving cell and/or a NES-mode indication field for the PCell

-	if the UE is configured with both cell DTX operation and cell DRX operation for the serving cell and if cellDTXDRX-L1activation is provided, the cell DTX/DRX indication field includes two bits where the first bit indicates the cell DTX operation and the second bit indicates the cell DRX operation

-	if the UE is configured with only one of the cell DTX operation and cell DRX operation for the serving cell and if cellDTXDRX-L1activation is provided, the cell DTX/DRX indication field includes one bit indicating one of the cell DTX operation and cell DRX operation, respectively, for the serving cell

-	a '0' value for a bit of the cell DTX/DRX indication field indicates deactivation of cell DTX or of cell DRX

-	a '1' value for a bit of the cell DTX/DRX indication field indicates activation of cell DTX or of cell DRX

-	if the serving cell is configured with a SUL carrier, the cell DTX/DRX indication field indication for activation or deactivation of cell DRX applies to both the UL carrier and the SUL carrier

-	if nesEvent is configured, the NES-mode indication field includes one bit indicating NES-specific CHO execution condition, as described in [12, TS 38.331]

-	a '0' value for the NES-mode indication field indicates NES-specific CHO execution condition is disabled

-	a '1' value for the NES-mode indication field, indicates NES-specific CHO execution condition is enabled

A UE does not expect to monitor PDCCH for detection of DCI format 2_9 on more than one serving cells of one cell group.

When a UE receives in slot m on the active DL BWP of a first serving cell a PDCCH providing DCI format 2_9 that indicates a change in activation or deactivation of a current cell DTX operation or cell DRX operation for a second serving cell, the UE operates on the second serving cell according to the indicated cell DTX operation or cell DRX operation starting from a slot on the active DL BWP or on the active UL BWP of the second serving cell, respectively, that is not before the beginning of the slot m+d on the active DL BWP of the first serving cell where d is a number of slots for the SCS of the active DL BWP of the first serving cell in Table 11.5-1.

Table 11.5-1: Minimum time gap value d

| SCS (kHz) | Number of slots |
| --- | --- |
| 15 | 3 |
| 30 | 6 |
| 60 | 12 |
| 120 | 24 |
| 480 | 96 |
| 960 | 192 |

[TS 38.321, clause 5.34.1]

Each Serving Cell may be configured by RRC with a periodic cell DTX pattern (i.e., Active and Non-Active Periods). The cell DTX operation controls UE's monitoring activity of PDCCH and configured downlink assignments in RRC_CONNECTED. For all activated Serving Cells with cell DTX configured and activated, the MAC entity may monitor PDCCH and configured downlink assignments using the cell DTX operation specified in clause 5.34.2.

Each Serving Cell may be configured by RRC with a periodic cell DRX pattern (i.e., Active and Non-Active Periods). The cell DRX operation controls Scheduling Request and configured uplink grant transmission activity in RRC_CONNECTED. For all activated Serving Cells with cell DRX configured and activated, the MAC entity may transmit configured uplink grant transmissions and Scheduling Request using the cell DRX operation specified in clause 5.34.3.

RRC controls cell DTX and cell DRX operation by configuring the following parameters in cellDTX-DRX-Config per Serving Cell:

-	cellDTX-DRX-ConfigType: defines whether only cell DTX is configured, only cell DRX is configured, or both are configured;

-	cellDTX-DRX-onDurationTimer: the active duration at the beginning of a cell DTX/DRX cycle;

-	cellDTX-DRX-StartOffset: defines the subframe where the cell DTX/DRX cycle starts;

-	cellDTX-DRX-SlotOffset: the delay before starting the cellDTX-DRX-onDurationTimer;

-	cellDTX-DRX-Cycle: the cell DTX/DRX cycle period.

-	ActivationStatus: the initial activation status of cell DTX and cell DRX operation.

[TS 38.321, clause 5.34.3]

Cell DRX is configured if cellDTX-DRX-ConfigType is set to drx or dtxdrx. Cell DRX operation is activated and deactivated for each Serving Cell by:

-	receiving a cell DRX indication from lower layers indicating activation or deactivation of cell DRX operation, as specified in TS 38.213 [6];

-	configuring cellDTX-DRX-Config by upper layers: if cell DRX is configured and ActivationStatus is set to activated, cell DRX operation is activated upon cell DRX configuration; if cell DRX is configured and -ActivationStatus is set to deactivated, cell DRX operation is deactivated upon cell DRX configuration; if cellDTX-DRX-Config is released, cell DRX operation is deactivated and all the corresponding configurations are released.

When cell DRX is configured and activated for a Serving Cell, the cell DRX Active Period includes the time while:

-	cellDTX-DRX-onDurationTimer is running for the associated Serving Cell.

For each Serving Cell configured with cell DRX, the MAC entity shall:

1>	if cell DRX is activated for this Serving Cell:

2>	if [(SFN × 10) + subframe number] modulo (cellDTX-DRX-Cycle) = (cellDTX-DRX-StartOffset):

3>	start cellDTX-DRX-onDurationTimer for this serving cell after cellDTX-DRX-SlotOffset from the beginning of the subframe.

1>	if cell DRX is activated and the Serving Cell is not in the cell DRX Active Period:

2>	not instruct the physical layer to signal a SR on a PUCCH resource for SR;

2>	not increment the SR_COUNTER for a SR;

2>	not start the sr-ProhibitTimer for a SR;

2>	not deliver any configured uplink grant and the associated HARQ information to the HARQ entity;

2>	not instruct a HARQ process associated with a configured uplink grant to trigger a new transmission or a retransmission;

2>	not report CSI on PUCCH and semi-persistent CSI configured on PUSCH;

2>	if an emergency service is initiated by upper layers and this Serving Cell is the SpCell:

3>	initiate a Random Access procedure (as specified in clause 5.1.1).

NOTE:	How the MAC layer in the UE is aware of an ongoing emergency service is up to UE implementation.

7.1.1.14.1.4.3	Test description

7.1.1.14.1.4.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0.

7.1.1.14.1.4.3.2	Test procedure sequence

Table 7.1.1.14.1.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits an RRCReconfiguration message to configure Cell DRX pattern. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | The SS transmits DCI format 2_9 indicating to activate cell DRX. (Note 1) | <-- | PDCCH (DCI 2-9) | - | - |
| 4 | Wait 5ms to ensure DCI format 2_9 indication take effects. | - | - | - | - |
| 4A | The SS transmits a MAC PDU. | <-- | MAC PDU | - | - |
| 5 | Check: Does the UE repeatedly transmit a Scheduling Request on the all available PUCCH when the Serving Cell is not in the cell DRX Active Period within 3s? | --> | SR | 1 | F |
| 6 | The SS transmits DCI 2-9 before the start of next cellDTX-DRX-onDurationTimer and the DCI 2-9 indicates to deactivate cell DRX. (Note 2) | <-- | PDCCH (DCI 2-9) | - | - |
| 7 | Wait 5ms to ensure DCI 2-9 indication take effects. | - | - | - | - |
| 8 | Check: Does the UE repeatedly transmit a Scheduling Request on the any available PUCCH when the Serving Cell is not in the cell DRX Active Period within 100ms? Note: The default period for SR is 10ms and default sr-TransMax is 16. 100ms < 16*10ms, so the SR_COUNTER is less than sr-TransMax. | --> | SR | 2 | P |
| 9 | The SS transmits an UL grant to allocate UL-SCH resources that are enough to transmit looped back PDU | <-- | (UL Grant ) | - | - |
| 10 | Check: Does the UE transmit a MAC PDU? | --> | MAC PDU | - | - |
| Note 1: The cell DTX/DRX indication field is value '1' in the DCI 2-9. Note 2: The cell DTX/DRX indication field is value '0' in the DCI 2-9. |  |  |  |  |  |

7.1.1.14.1.4.3.3	Specific message contents

Table 7.1.1.14.1.4.3.3-1: RRCReconfiguration (step 1, Table 7.1.1.14.1.4.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| secondaryCellGroup |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig | Table 7.1.1.14.1.4.3.3-2 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.14.1.4.3.3-2: CellGroupConfig (Table 7.1.1.14.1.4.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |  |
| physicalCellGroupConfig |  | PhysicalCellGroupConfig | Table 7.1.1.14.1.4.3.3-3 |  |
| spCellConfig SEQUENCE { |  |  |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |  |
| servingCellConfig |  | ServingCellConfig | Table 7.1.1.14.1.4.3.3-4 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.14.1.4.3.3-3: PhysicalCellGroupConfig (Table 7.1.1.14.1.4.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PhysicalCellGroupConfig::= SEQUENCE { |  |  |  |  |
| cellDTRX-DCI-config-r18 CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| cellDTRX-RNTI-r18 |  | RNTI-Value |  |  |
| sizeDCI-2-9-r18 |  | 1 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.14.1.4.3.3-4: ServingCellConfig (Table 7.1.1.14.1.4.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| ServingCellConfig SEQUENCE { |  |  |  |  |
| initialDownlinkBWP ::= SEQUENCE { |  |  |  |  |
| pdcch-Config CHOICE { |  |  |  |  |
| setup |  | PDCCH-Config | Table 7.1.1.14.1.4.3.3-5 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| cellDTX-DRX-Config-r18 CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| cellDTX-DRX-onDurationTimer-r18 CHOICE { |  |  |  |  |
| milliSeconds |  | ms1 |  |  |
| } |  |  |  |  |
| cellDTX-DRX-CycleStartOffset-r18 CHOICE { |  |  |  |  |
| ms640 |  | 0 |  |  |
| } |  |  |  |  |
| cellDTX-DRX-SlotOffset-r18 |  | 0 |  |  |
| cellDTX-DRX-ConfigType-r18 |  | drx |  |  |
| cellDTX-DRX-ActivationStatus-r18 |  | deactivated |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| positionInDCI-cellDTRX-r18 |  | 0 |  |  |
| cellDTX-DRX-L1activation-r18 |  | enabled |  |  |
| } |  |  |  |  |

Table 7.1.1.14.1.4.3.3-5: PDCCH-Config (Table 7.1.1.14.1.4.3.3-4)

Derivation Path: TS 38.508-1 [4], Table 4.6.3-95 with condition DCI_2_9

##### 7.1.1.14.2	NES spatial domain adaptations

###### 7.1.1.14.2.1		NES spatial domain Type 1 adaptation / Periodic CSI reporting

7.1.1.14.2.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state configured for Periodic CSI-RS measurement reporting }

ensure that {

when { UE receives RRCReconfiguration message to report periodical CSI measurements with CSI-ReportSubconfig along with minimum 2 CSI-ReportSubconfigID }

then { UE applies CSI-ReportConfig and sends periodical CSI measurement reports for all CSI-ReportSubconfig present in CSI-ReportConfig }

}

7.1.1.14.2.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.212, clause 6.3.1.1.2 and TS 38.214, clauses 5.2.1.4.2 and 5.2.4. Unless otherwise stated these are Rel-18 requirements.

[TS 38.212, clause 6.3.1.1.2]

…

If CSI-ReportSubConfig is configured, for a corresponding CSI sub-report, the bitwidth of a CSI field of the CSI sub-report is determined following the procedure in this clause 6.3.1.1.2 by taking configurations in CSI-ReportSubConfig when applicable. If CSI-ReportSubConfig configures a list of CSI-RS resource IDs, for the determination of the bitwidth of a CRI field, the value of KsCSI-RS is the number of CSI-RS resources configured in the corresponding CSI-ReportSubConfig.

…

[TS 38.214, clause 5.2.1.4.2]

…

If the UE is configured with a CSI-ReportConfig that contains a list of sub-configurations, provided by csi-ReportSubConfigToAddModList:

-	The UE expects to be configured with the higher layer parameter codebookType set to 'typeI-SinglePanel' or 'typeI-MultiPanel'. If the UE indicates a capability for supporting mixed codebook combination in a slot with mixCodeBookSpatialAdaptation, each sub-configuration which is configured with portSubsetIndictor can be configured with the higher layer parameter codebookType set to 'typeI-SinglePanel' or 'typeI-MultiPanel'.

-	Each sub-configuration can be configured with an antenna port subset using the higher layer bitmap parameter portSubsetIndicator which contains the bit sequence p0,p1,...,pPm-1, where p0 is the MSB and pPm-1 is the LSB, bit pi corresponds to antenna port 3000+i, and Pm is the number of ports nrofPorts configured for the CSI-RS resources(s) within a NZP-CSI-RS-ResourceSet contained in the CSI-ResourceConfig for channel measurement that corresponds to the CSI-ReportConfig. A bit value 0 in portSubsetIndicator indicates that the corresponding antenna port is disabled for the sub-configuration, whereas bit value 1 indicates that the antenna port is enabled and belongs to the antenna port subset for the sub-configuration. For the derivation of PMI, antenna ports corresponding to all bits with value of 1 in portSubsetIndicator are mapped to consecutive antenna ports starting at CSI-RS antenna port 3000 in increasing order of the bit position in portSubsetIndicator.

-	If a sub-configuration is configured with an antenna port subset, then the sub-configuration can be configured with a ri-Restriction, codebookMode and, if the number of antenna ports of the subset greater than 2, with n1-n2 if the higher layer parameter codebookType is set to 'typeI-SinglePanel' or with ng-n1-n2 if the higher layer parameter codebookType is set to 'typeI-MultiPanel', and, if the corresponding number of antenna ports of the subset is 2, with twoTX-CodebookSubsetRestriction, where the parameters ri-Restriction, codebookMode, n1-n2, ng-n1-n2, twoTX-CodebookSubsetRestriction are as described in Clauses 5.2.2.2.1 and 5.2.2.2.2. If a sub-configuration is configured with an antenna port subset, and if higher layer parameter reportQuantity is set to 'cri-RI-i1-CQI', and if the higher layer parameter codebookType is set to 'typeI-SinglePanel', then the sub-configuration is configured with higher layer parameter typeI-SinglePanel-codebookSubsetRestriction-i2, where typeI-SinglePanel-codebookSubsetRestriction-i2 is as described in Clause 5.2.2.2.1.

-	If a sub-configuration is configured with an antenna port subset, and if the CSI-ReportConfig that contains a mix of sub-configuration(s) each corresponding to 'typeI-SinglePanel' and some other sub-configuration(s) each corresponding to 'typeI-MultiPanel', then the sub-configuration(s) which is configured with portSubsetIndictor is configured with the higher layer parameter codebookMode.

-	A sub-configuration can be configured with a power offset provided by powerOffset.

-	A sub-configuration can be configured with a list of NZP CSI-RS resources, provided by nzp-CSI-RS-ResourceList, which indicates one or more NZP CSI-RS resources, within a NZP-CSI-RS-ResourceSet contained in the CSI-ResourceConfig for channel measurement which corresponds to the CSI-ReportConfig. If there is no sub-configuration configured with a power offset provided by powerOffset, the list of NZP CSI-RS resources has no intersection with a list of NZP CSI-RS resources configured for any other sub-configuration(s) within the CSI-ReportConfig, otherwise, the list of NZP CSI-RS resources is identical to or has no intersection with a list of NZP CSI-RS resources configured for any other sub-configuration(s) within the CSI-ReportConfig.

-	If a sub-configuration is configured with a list of NZP CSI-RS resources with more than one resource, the UE shall derive the CSI parameters other than CRI conditioned on the reported CRI, where the CRI k (k ≥ 0) for the sub-configuration corresponds to the configured (k+1)-th entry of associated NZP-CSI-RS-Resource in the list of NZP CSI-RS resources.

-	If a sub-configurations is not configured with nzp-CSI-RS-ResourceList then the sub-configuration shall be associated with all the NZP CSI-RS resources within a NZP-CSI-RS-ResourceSet contained in the CSI-ResourceConfig for channel measurement which corresponds to the CSI-ReportConfig.

-	the UE reports CSI(s) for one or more sub-configurations according to Clauses 5.2.1.5.1, 5.2.1.5.2, 5.2.3 and 5.2.4, and according to the higher layer parameter reportQuantity configured for that CSI-ReportConfig.

-	The UE does not expect the higher layer parameter reportQuantity to be set to 'cri-RSRP', 'cri-SINR', 'cri-SINR- Index', 'cri-RSRP-Index', 'none', 'ssb-Index-RSRP', 'ssb-Index-SINR', 'ssb-Index-RSRP- Index', 'ssb-Index-SINR- Index', or 'tdcp'.

…

[TS 38.214, clause 5.2.4]

A UE is semi-statically configured by higher layers to perform periodic CSI Reporting on the PUCCH. A UE can be configured by higher layers for multiple periodic CSI Reports corresponding to multiple higher layer configured CSI Reporting Settings, where the associated CSI Resource Settings are higher layer configured. For a Reporting Setting for which the CSI-ReportConfig contains a list of sub-configurations provided by csi-ReportSubConfigToAddModList, CSI reporting is provided for all the sub-configurations in each corresponding reporting instance. Periodic CSI reporting on PUCCH formats 2, 3, 4 supports Type I CSI with wideband granularity.

7.1.1.14.2.1.3	Test description

7.1.1.14.2.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

UE:

- None.

Preamble:

-	The UE is in state 3N-A as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1.

7.1.1.14.2.1.3.2	Test procedure sequence

Table 7.1.1.14.2.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message on NR Cell 1 including CSI-MeasConfig to setup measurement and periodic reporting with CSI-ReportSubconfig. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message on NR Cell 1. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | Check: Does the UE transmit CSI report on PUCCH for all CSI-ReportSubconfig configured at a timing occasion according to CSI-ReportPeriodicityAndOffset? | --> | CSI Report on PUCCH | 1 | P |
| 4 | Check: Does the UE transmit CSI report on PUCCH for all CSI-ReportSubconfig configured at a timing occasion according to CSI-ReportPeriodicityAndOffset? | --> | CSI Report on PUCCH | 1 | P |
| 5 | The SS transmits an RRCReconfiguration including CSI-MeasConfig to remove measurements | <-- | NR RRC: RRCReconfiguration | - | - |
| 6 | The UE transmits an RRCReconfigurationComplete message to confirm the remove. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 7 | Check: Does the UE transmit CSI report on PUCCH for all CSI-ReportSubconfig configured within two timing occasion according to CSI-ReportPeriodicityAndOffset? | --> | CSI Report on PUCCH | 1 | F |

7.1.1.14.2.1.3.3	Specific message contents

Table 7.1.1.14.2.1.3.3-1: RRCReconfiguration (Step 1 and Step 5, Table 7.1.1.14.2.1.3.2-1)

| Derivation Path: TS 38.508-1 [4] table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig with condition MEAS | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.14.2.1.3.3-2: ServingCellConfig (Table 7.1.1.14.2.1.3.3-1)

| Derivation Path: TS 38.508-1 [4], table 4.6.3-167 with condition MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| csi-MeasConfig | CSI-MeasConfig |  |  |
| servingCellMO | Not present |  |  |
| } |  |  |  |

Table 7.1.1.14.2.1.3.3-3: CSI-MeasConfig (Table 7.1.1.14.2.1.3.3-2)

Derivation Path: TS 38.508-1 [4], table 4.6.3-38

Table 7.1.1.14.2.1.3.3-4: CSI-ReportConfig (Step 1, Table 7.1.1.14.2.1.3.3-3)

| Derivation Path: TS 38.508-1 [4], table 4.6.3-39 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| reportConfigType CHOICE { |  |  |  |
| periodic SEQUENCE { |  |  |  |
| reportSlotConfig | CSI-ReportPeriodicityAndOffset | Table 4.6.3-40D in TS 38.508-1 [4] |  |
| pucch-CSI-ResourceList SEQUENCE (SIZE (1.. maxNrofBWPs)) OF PUCCH-CSI-Resource { |  |  |  |
| PUCCH-CSI-Resource[1] | PUCCH-CSI-Resource | Table 4.6.3-113B in TS 38.508-1 [4] |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| csi-ReportSubConfigToAddModList-r18 SEQUENCE (SIZE (1..maxNrofCSI-ReportSubconfigPerCSI-ReportConfig-r18)) OF CSI-ReportSubConfig-r18 { | 2 entries |  |  |
| CSI-ReportSubConfig-r18[1] SEQUENCE{ |  | Entry 1 |  |
| reportSubConfigId-r18 | 0 |  |  |
| reportSubConfigParams-r18 CHOICE { |  |  |  |
| a1-parameters SEQUENCE { |  |  |  |
| codebookSubConfig-r18 | CodebookConfig | Table 4.6.3-25 in TS 38.508-1 [4] |  |
| portSubsetIndicator-r18 CHOICE { |  |  |  |
| p8 | ‘55’H | BIT STRING (SIZE(8)) | FR1 |
| p4 | ‘5’H | BIT STRING (SIZE(4)) | FR2 |
| } |  |  |  |
| non-PMI-PortIndication-r18 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| powerOffset-r18 | Not present |  |  |
| } |  |  |  |
| CSI-ReportSubConfig-r18[2] SEQUENCE{ |  | Entry 2 |  |
| reportSubConfigId-r18 | 1 |  |  |
| reportSubConfigParams-r18 CHOICE { |  |  |  |
| a1-parameters SEQUENCE { |  |  |  |
| codebookSubConfig-r18 | CodebookConfig | Table 4.6.3-25 in TS 38.508-1 [4] |  |
| portSubsetIndicator-r18 CHOICE { |  |  |  |
| p8 | ‘AA’H | BIT STRING (SIZE(8)) | FR1 |
| p4 | ‘A’H | BIT STRING (SIZE(4)) | FR2 |
| } |  |  |  |
| non-PMI-PortIndication-r18 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| powerOffset-r18 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.14.2.1.3.3-5: CSI-ReportConfig (Step 5, Table 7.1.1.14.2.1.3.3-3)

| Derivation Path: TS 38.508-1 [4], table 4.6.3-39 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| csi-ReportSubConfigToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofCSI-ReportSubconfigPerCSI-ReportConfig-r18)) OF CSI-ReportSubConfigId-r18 { | 2 entries |  |  |
| CSI-ReportSubConfigId-r18[1] | 0 | Entry 1 |  |
| CSI-ReportSubConfigId-r18[2] | 1 | Entry 2 |  |
| } |  |  |  |
| } |  |  |  |

###### 7.1.1.14.2.2	NES spatial domain Type 2 adaptation / Aperiodic CSI reporting

7.1.1.14.2.2.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and supporting spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s) for aperiodic CSI reporting }

ensure that {

when { UE receives RRCReconfiguration message with CSI-ReportConfig that contains a list of sub-configurations each corresponding to a list of one or more CSI-RS resources }

then { UE performs aperiodic CSI reporting on PUSCH as indicated by CSI triggering state for aperiodic CSI reporting }

}

7.1.1.14.2.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.214, clauses 5.2.1.4.2, 5.2.1.5.1 and 5.2.3. Unless otherwise stated these are Rel-18 requirements.

[TS 38.214, clause 5.2.1.4.2]

…

If the UE is configured with a CSI-ReportConfig that contains a list of sub-configurations, provided by csi-ReportSubConfigToAddModList:

…

-	A sub-configuration can be configured with a list of NZP CSI-RS resources, provided by nzp-CSI-RS-ResourceList, which indicates one or more NZP CSI-RS resources, within a NZP-CSI-RS-ResourceSet contained in the CSI-ResourceConfig for channel measurement which corresponds to the CSI-ReportConfig. If there is no sub-configuration configured with a power offset provided by powerOffset, the list of NZP CSI-RS resources has no intersection with a list of NZP CSI-RS resources configured for any other sub-configuration(s) within the CSI-ReportConfig, otherwise, the list of NZP CSI-RS resources is identical to or has no intersection with a list of NZP CSI-RS resources configured for any other sub-configuration(s) within the CSI-ReportConfig.

-	If a sub-configuration is configured with a list of NZP CSI-RS resources with more than one resource, the UE shall derive the CSI parameters other than CRI conditioned on the reported CRI, where the CRI k (k ≥ 0) for the sub-configuration corresponds to the configured (k+1)-th entry of associated NZP-CSI-RS-Resource in the list of NZP CSI-RS resources.

-	If a sub-configurations is not configured with nzp-CSI-RS-ResourceList then the sub-configuration shall be associated with all the NZP CSI-RS resources within a NZP-CSI-RS-ResourceSet contained in the CSI-ResourceConfig for channel measurement which corresponds to the CSI-ReportConfig.

-	the UE reports CSI(s) for one or more sub-configurations according to Clauses 5.2.1.5.1, 5.2.1.5.2, 5.2.3 and 5.2.4, and according to the higher layer parameter reportQuantity configured for that CSI-ReportConfig.

-	The UE does not expect the higher layer parameter reportQuantity to be set to 'cri-RSRP', 'cri-SINR', 'cri-SINR- Index', 'cri-RSRP-Index', 'none', 'ssb-Index-RSRP', 'ssb-Index-SINR', 'ssb-Index-RSRP- Index', 'ssb-Index-SINR- Index', or 'tdcp'.

…

[TS 38.214, clause 5.2.1.5.1]

For CSI-RS resource sets associated with Resource Settings configured with the higher layer parameter resourceType set to 'aperiodic', 'periodic', or 'semi-persistent', trigger states for Reporting Setting(s) (configured with the higher layer parameter reportConfigType set to 'aperiodic') and/or Resource Setting for channel and/or interference measurement on one or more component carriers are configured using the higher layer parameter CSI-AperiodicTriggerStateList. For a reporting setting for which the CSI-ReportConfig contains a list of sub-configurations provided by the higher layer parameter csi-ReportSubConfigToAddModList, one or more trigger states can be configured with each indicating one or more of the sub-configurations. For aperiodic CSI report triggering, a single set of CSI triggering states are higher layer configured, wherein the CSI triggering states can be associated with any candidate DL BWP.

[TS 38.214, clause 5.2.3]

A UE shall perform aperiodic CSI reporting using PUSCH on serving cell c upon successful decoding of a DCI format 0_1 or DCI format 0_2 which triggers an aperiodic CSI trigger state. A UE shall perform aperiodic CSI reporting using PUSCH on the serving cell with the smallest serving cell index scheduled by DCI format 0_3 which triggers an aperiodic CSI trigger state.

7.1.1.14.2.2.3	Test description

7.1.1.14.2.2.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used in NR cell 1.

UE:

-	None.

Preamble:

-	The UE is in state 3N-A as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1.

7.1.1.14.2.2.3.2	Test procedure sequence

Table 7.1.1.14.2.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message to configure CSI-ReportConfig containing a list of sub-configurations and CSI-ReportSubConfigTriggerList. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | The SS transmits MAC control element to indicate aperiodic CSI trigger State. | <-- | MAC PDU (Aperiodic CSI Trigger State Subselection MAC CE (T0=1)) | - | - |
| 4 | The SS transmits a downlink assignment to trigger aperiodic CSI reporting for sub-configuration #1. (Note 1) | <-- | DCI format 0_1 | - | - |
| 5 | Check: Does the UE transmit 1 CSI report within 20ms? | --> | PUSCH (CSI report) | 1 | P |
| 6 | The SS transmits MAC control element to indicate aperiodic CSI trigger State. | <-- | MAC PDU (Aperiodic CSI Trigger State Subselection MAC CE (T1=1)) | - | - |
| 7 | The SS transmits a downlink assignment to trigger aperiodic CSI reporting for both sub-configuration #1 and sub-configuration #2. (Note 1) | <-- | DCI format 0_1 | - | - |
| 8 | Check: Does the UE transmit 2 CSI reports within 20ms? | --> | PUSCH (2 CSI reports) | 1 | P |
| Note 1: The CSI request field is value ‘1' in the DCI format 0_1. |  |  |  |  |  |

7.1.1.14.2.2.3.3	Specific message contents

Table 7.1.1.14.2.2.3.3-1: RRCReconfiguration (Step 1, Table 7.1.1.14.2.2.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig | Table 7.1.1.14.2.2.3.3-2 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.14.2.2.3.3-2: CellGroupConfig

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-19 with condition MEAS |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| CellGroupConfig ::= SEQUENCE { |  |  |  |  |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |  |  |  |  |
| spCellConfigDedicated |  | ServingCellConfig |  | Table 7.1.1.14.2.2.3.3-3 |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 7.1.1.14.2.2.3.3-3: ServingCellConfig

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-167 with condition MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| csi-MeasConfig | CSI-MeasConfig | Table 7.1.1.14.2.2.3.3-4 |  |
| servingCellMO | Not present |  |  |
| } |  |  |  |

Table 7.1.1.14.2.2.3.3-4: CSI-MeasConfig

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-38 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-MeasConfig::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource { | 2 entries |  |  |
| NZP-CSI-RS-Resource[1] | NZP-CSI-RS-Resource-1 | entry 1, CSI-RS resource 1 for sub-configuration #1 |  |
| NZP-CSI-RS-Resource[2] | NZP-CSI-RS-Resource-2 | entry 2, CSI-RS resource 2 for sub-configuration #2 |  |
| } |  |  |  |
| nzp-CSI-RS-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSetId { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSet[1] | NZP-CSI-RS-ResourceSet | entry 1, Table 7.1.1.14.2.2.3.3-5 |  |
| } |  |  |  |
| csi-SSB-ResourceSetToAddModList | Not present |  |  |
| csi-ReportConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfig { | 1 entry |  |  |
| CSI-ReportConfig[1] | CSI-ReportConfig | entry 1, Table 7.1.1.14.2.2.3.3-8 |  |
| } |  |  |  |
| reportTriggerSize | 1 |  |  |
| aperiodicTriggerStateList CHOICE { |  |  |  |
| setup | CSI-AperiodicTriggerStateList | Table 7.1.1.14.2.2.3.3-9 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.14.2.2.3.3-5: NZP-CSI-RS-ResourceSet (Table 7.1.1.14.2.2.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-87 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSet ::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 2 entries |  |  |
| NZP-CSI-RS-ResourceId[1] | 0 | entry 1, CSI-RS resource 1 |  |
| NZP-CSI-RS-ResourceId[2] | 1 | entry 2, CSI-RS resource 2 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.14.2.2.3.3-6: NZP-CSI-RS-Resource-1

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-85 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceId | 0 | CSI-RS resource 1 |  |
| resourceMapping | CSI-RS-ResourceMapping | Table 7.1.1.14.2.2.3.3-7 |  |
| } |  |  |  |

Table 7.1.1.14.2.2.3.3-6A: NZP-CSI-RS-Resource-2

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-85 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceId | 1 | CSI-RS resource 2 |  |
| resourceMapping | CSI-RS-ResourceMapping-2 | Table 7.1.1.14.2.2.3.3-7A |  |
| } |  |  |  |

Table 7.1.1.14.2.2.3.3-7: CSI-RS-ResourceMapping-1

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| firstOFDMSymbolInTimeDomain | 3 | l0 = 3 for CSI-RS resource 1 | FR1 |
|  | 11 | l0 = 11 for CSI-RS resource 1 | FR2 |
| } |  |  |  |

Table 7.1.1.14.2.2.3.3-7A: CSI-RS-ResourceMapping-2

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| firstOFDMSymbolInTimeDomain | 7 | l0 = 7 for CSI-RS resource 2 | FR1 |
|  | 13 | l0 = 13 for CSI-RS resource 2 | FR2 |
| } |  |  |  |

Table 7.1.1.14.2.2.3.3-8: CSI-ReportConfig (Table 7.1.1.14.2.2.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-39 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| csi-ReportSubConfigToAddModList-r18 SEQUENCE (SIZE (1..maxNrofCSI-ReportSubconfigPerCSI-ReportConfig-r18)) OF CSI-ReportSubConfig-r18 { | 2 entries |  |  |
| CSI-ReportSubConfig-r18[1] SEQUENCE { |  | entry 1 |  |
| reportSubConfigId-r18 | 0 | sub-configuration #1 |  |
| reportSubConfigParams-r18 CHOICE { |  |  |  |
| a2-parameters SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceList-r18 SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceIndex-r18 { | 1 entry |  |  |
| NZP-CSI-RS-ResourceIndex-r18[1] | 0 | entry 1, CSI-RS resource 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| powerOffset-r18 | Not present |  |  |
| } |  |  |  |
| CSI-ReportSubConfig-r18[2] SEQUENCE { |  | entry 2 |  |
| reportSubConfigId-r18 | 1 | sub-configuration #2 |  |
| reportSubConfigParams-r18 CHOICE { |  |  |  |
| a2-parameters SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceList-r18 SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceIndex-r18 { | 1 entry |  |  |
| NZP-CSI-RS-ResourceIndex-r18[1] | 1 | entry 1, CSI-RS resource 2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| powerOffset-r18 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.14.2.2.3.3-9: CSI-AperiodicTriggerStateList (Table 7.1.1.14.2.2.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-32 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-AperiodicTriggerStateList ::= SEQUENCE (SIZE (1..maxNrOfCSI-AperiodicTriggers)) OF CSI-AperiodicTriggerState { | 2 entries |  |  |
| CSI-AperiodicTriggerState[1] SEQUENCE { |  | entry 1 |  |
| associatedReportConfigInfoList SEQUENCE (SIZE(1..maxNrofReportConfigPerAperiodicTrigger)) OF CSI-AssociatedReportConfigInfo { | 1 entry |  |  |
| CSI-AssociatedReportConfigInfo[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 0 |  |  |
| resourcesForChannel CHOICE { |  |  |  |
| nzp-CSI-RS SEQUENCE { |  |  |  |
| resourceSet | 1 |  |  |
| qcl-info SEQUENCE (SIZE(1..maxNrofAP-CSI-RS-ResourcesPerSet)) OF TCI-StateId { | 1 entry |  |  |
| TCI-StateId[1] | 0 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| csi-IM-ResourcesforInteference | Not present |  |  |
| nzp-CSI-RS-ResourcesforInterference | Not present |  |  |
| csi-ReportSubConfigTriggerList-r18 SEQUENCE (SIZE(1..maxNrofCSI-ReportSubconfigPerCSI-ReportConfig-r18)) OF CSI-ReportSubConfigId-r18 { | 1 entry |  |  |
| CSI-ReportSubConfigId-r18[1] | 0 | sub-configuration #1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| CSI-AperiodicTriggerState[2] SEQUENCE { |  | entry 2 |  |
| associatedReportConfigInfoList SEQUENCE (SIZE(1..maxNrofReportConfigPerAperiodicTrigger)) OF CSI-AssociatedReportConfigInfo { | 1 entry |  |  |
| CSI-AssociatedReportConfigInfo[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 0 |  |  |
| resourcesForChannel CHOICE { |  |  |  |
| nzp-CSI-RS SEQUENCE { |  |  |  |
| resourceSet | 1 |  |  |
| qcl-info SEQUENCE (SIZE(1..maxNrofAP-CSI-RS-ResourcesPerSet)) OF TCI-StateId { | 1 entry |  |  |
| TCI-StateId[1] | 0 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| csi-IM-ResourcesforInteference | Not present |  |  |
| nzp-CSI-RS-ResourcesforInterference | Not present |  |  |
| csi-ReportSubConfigTriggerList-r18 SEQUENCE (SIZE(1..maxNrofCSI-ReportSubconfigPerCSI-ReportConfig-r18)) OF CSI-ReportSubConfigId-r18 { | 2 entries |  |  |
| CSI-ReportSubConfigId-r18[1] | 0 | entry 1, sub-configuration #1 |  |
| CSI-ReportSubConfigId-r18[2] | 1 | entry 2, sub-configuration #2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.14.2.2.3.3-10: PUSCH-Config

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-118 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| pusch-TimeDomainAllocationList CHOICE { |  |  |  |
| setup { |  |  |  |
| PUSCH-TimeDomainResourceAllocationList SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF PUSCH-TimeDomainResourceAllocation { | 1 entry |  |  |
| PUSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  | Entry 1 |  |
| k2 | 4 |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 27 | Start symbol(S)=0, Length(L)=14 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

###### 7.1.1.14.2.3	NES Power domain adaptation / Semi-Persistent CSI reporting

7.1.1.14.2.3.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state configured for Semi-Persistent CSI-RS measurement reporting }

ensure that {

when { UE receives RRCReconfiguration message to report Semi-Persistent CSI measurements with CSI-ReportSubconfig with multiple CSI-ReportSubconfigID }

then { UE applies CSI-ReportConfig and sends Semi-Persistent CSI measurement reports for all CSI-ReportSubconfig present in CSI-ReportConfig }

}

7.1.1.14.2.3.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.214, clauses 5.2.1.5.2, 5.2.4 and TS 38.321 clause 6.1.3.80. Unless otherwise stated these are Rel-18 requirements.

[TS 38.214, clause 5.2.1.5.2]

…

For semi-persistent reporting on PUCCH, the PUCCH resource used for transmitting the CSI report are configured by reportConfigType. Semi-persistent reporting on PUCCH is activated by an activation command as described in clause 6.1.3.16 of [10, TS 38.321], which selects one of the semi-persistent Reporting Settings for use by the UE on the PUCCH. For a selected reporting setting for which the CSI-ReportConfig contains a list of sub-configurations provided by the higher layer parameter csi-ReportSubConfigToAddModList, the activation command can also select one or more sub-configurations to use by the UE as described in clause 6.1.3.16 of [10, TS 38.321]. When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, the indicated semi-persistent Reporting Setting should be applied starting from the first slot that is after slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH

…

[TS 38.214, clause 5.2.4]

…

A UE shall perform semi-persistent CSI reporting on the PUCCH applied starting from the first slot that is after slot n+3Nslotsubframe,µ when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command described in clause 6.1.3.16 of [10, TS 38.321] where  is the SCS configuration for the PUCCH. The activation command will contain one or more Reporting Settings, with or without containing one or more sub-configurations for each Reporting Setting for which the CSI-ReportConfig contains a list of sub-configurations provided by csi-ReportSubConfigToAddModList, where the associated CSI Resource Settings are configured. Semi-persistent CSI reporting on the PUCCH supports Type I CSI. Semi-persistent CSI reporting on the PUCCH format 2 supports Type I CSI with wideband frequency granularity. Semi-persistent CSI reporting on PUCCH formats 3 or 4 supports Type I CSI with wideband and sub-band frequency granularities and Type II CSI Part 1

…

[TS 38.321, clause 6.1.3.80]

The enhanced SP CSI reporting on PUCCH Activation/Deactivation MAC CE is identified by a MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a variable size and consists of the following fields:

-	Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits;

-	BWP ID: This field indicates a UL BWP for which the MAC CE applies as the codepoint of the DCI bandwidth part indicator field as specified in TS 38.212 [9]. The length of the BWP ID field is 2 bits;

-	Si: This field indicates the activation/deactivation status of the Semi-Persistent CSI report configuration within csi-ReportConfigToAddModList, as specified in TS 38.331 [5]. S0 refers to the report configuration which includes PUCCH resources for SP CSI reporting in the indicated BWP and has the lowest CSI-ReportConfigId within the list with type set to semiPersistentOnPUCCH, S1 to the report configuration which includes PUCCH resources for SP CSI reporting in the indicated BWP and has the second lowest CSI-ReportConfigId and so on. If the number of report configurations within the list with type set to semiPersistentOnPUCCH in the indicated BWP is less than i + 1, MAC entity shall ignore the Si field. The Si field is set to 1 to indicate that the corresponding Semi-Persistent CSI report configuration shall be activated. The Si field is set to 0 to indicate that the corresponding Semi-Persistent CSI report configuration i shall be deactivated;

-	Ni,x: This field indicates the activation/deactivation status of the Semi-Persistent CSI report sub-configuration x within csi-ReportSubConfigToAddModList, as specified in TS 38.331 [5]. If Si set to 1, the octet corresponding to Ni,0 to Ni,7 is present. If Si set to 0, the octet corresponding to Ni,0 to Ni,7 is not present. N0,0 refers to the report sub-configuration which has the lowest csi-ReportSubConfigID within the list, N0,1 to the report sub-configuration which has the second lowest csi-ReportSubConfigID and so on. If the number of report sub-configurations within the list in the indicated CSI-ReportConfig is less than x + 1, the MAC entity shall ignore the Ni,x field. The Ni,x field is set to 1 to indicate that the corresponding Semi-Persistent CSI report sub-configuration x shall be activated. The Ni,x field is set to 0 to indicate that the corresponding Semi-Persistent CSI report sub-configuration x shall be deactivated;

-	R: Reserved bit, set to 0.

Figure 6.1.3.80-1: Enhanced SP CSI reporting on PUCCH Activation/Deactivation MAC CE

7.1.1.14.2.3.3	Test description

7.1.1.14.2.3.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

UE:

-	None.

Preamble:

-	The UE is in state 3N-A as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1.

7.1.1.14.2.3.3.2	Test procedure sequence

Table 7.1.1.14.2.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message on NR Cell 1 including CSI-MeasConfig to setup measurement and semiPersistentOnPUCCH in CSI-ReportSubconfig with different powerOffset-r18 | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message on NR Cell 1. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | The SS transmits a SP CSI reporting on PUCCH Activation MAC-CE | <-- | MAC PDU (S0=1 and N0,0=1, N0,1=1) | - | - |
| 4 | Check: Does the UE transmit CSI report on PUCCH for all CSI-ReportSubconfig configured in first periodicity? | --> | CSI report on PUCCH | 1 | P |
| 5 | Check: Does the UE transmit CSI report on PUCCH for all CSI-ReportSubconfig configured in second periodicity? | --> | CSI report on PUCCH | 1 | P |
| 6 | The SS transmits a SP CSI reporting on PUCCH Deactivation MAC-CE | <-- | MAC PDU | - | - |
| 7 | Check: Does the UE transmit CSI report on PUCCH for all CSI-ReportSubconfig configured within two periods? | --> | CSI report on PUCCH | 1 | F |
| 8 | The SS transmits an RRCReconfiguration including CSI-measConfig to remove measurements | <-- | NR RRC: RRCReconfiguration | - | - |
| 9 | The UE transmits an RRCReconfigurationComplete message to confirm the remove | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 10 | Check: Does the UE transmit CSI report on PUCCH for all CSI-ReportSubconfig configured within two periods? | --> | CSI report on PUCCH | 1 | F |

7.1.1.14.2.3.3.3	Specific message contents

Table 7.1.1.14.2.3.3.3-1: RRCReconfiguration (Step 1 and Step 8, Table 7.1.1.14.2.3.3.2-1)

Derivation Path: TS 38.508-1 [4] table 4.6.1-13 with condition NR_MEAS

Table 7.1.1.14.2.3.3.3-2: ServingCellConfig (Table 7.1.1.14.2.3.3.3-1)

| Derivation Path: TS 38.508-1 [4], clause 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| csi-MeasConfig | CSI-MeasConfig |  |  |
| } |  |  |  |

Table 7.1.1.14.2.3.3.3-3: CSI-MeasConfig (Table 7.1.1.14.2.3.3.3-2)

Derivation Path: TS 38.508-1 [4], clause 4.6.3-38

Table 7.1.1.14.2.3.3.3-4: CSI-ReportConfig (Step 1, Table 7.1.1.14.2.3.3.3-3)

| Derivation Path: TS 38.508-1 [4], clause 4.6.3-39 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| csi-ReportSubConfigToAddModList-r18 SEQUENCE (SIZE (1..maxNrofCSI-ReportSubconfigPerCSI-ReportConfig-r18)) OF CSI-ReportSubConfig-r18 { | 2 entries |  |  |
| CSI-ReportSubConfig-r18[1] SEQUENCE{ |  | Entry 1 |  |
| reportSubConfigId-r18 | 0 |  |  |
| reportSubConfigParams-r18 | Not present |  |  |
| powerOffset-r18 | 3 |  |  |
| } |  |  |  |
| CSI-ReportSubConfig-r18[2] SEQUENCE{ |  | Entry 2 |  |
| reportSubConfigId-r18 | 1 |  |  |
| reportSubConfigParams-r18 | Not present |  |  |
| powerOffset-r18 | 6 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.14.2.3.3.3-5: CSI-ReportConfig (Step 8, Table 7.1.1.14.2.3.3.3-3)

| Derivation Path: TS 38.508-1 [4], clause 4.6.3-39 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| csi-ReportSubConfigToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofCSI-ReportSubconfigPerCSI-ReportConfig-r18)) OF CSI-ReportSubConfigId-r18 { | 1 entry |  |  |
| CSI-ReportSubConfigId-r18[1] | 0 |  |  |
| CSI-ReportSubConfigId-r18[2] | 1 |  |  |
| } |  |  |  |
| } |  |  |  |

### 7.1.2	RLC

#### 7.1.2.1	Default Pre-Test Conditions for all RLC test cases

The following pre-test conditions shall be applied in all RLC test cases until the test case explicitly over writes these conditions.

##### 7.1.2.1.1	Default Pre-Test Conditions for AM RLC test cases

System Simulator:

-	The SS configures the test environment in accordance to the execution conditions in Table 7.1.2.1.1-1.

UE:

-	None

Preamble:

-	The SS performs the generic procedure in [4] to get UE in state RRC_CONNECTED in accordance to the execution conditions in Table 7.1.2.1.1-2 and the message condition UE TEST LOOP MODE A to return one UL PDCP SDU per DL PDCP SDU.

Table 7.1.2.1.1-1: Test environment

| Execution Condition | Cell configuration | System Information Combination |
| --- | --- | --- |
| IF pc_NG_RAN_NR | NR Cell 1 | NR: System information Combination NR-1 |
| ELSE IF pc_EN_DC | E-UTRA Cell 1 is PCell, NR Cell 1 is PSCell | EUTRA: System information Combination 1 NR: N/A |
| ELSE IF pc_NGEN_DC | NG-RAN E-UTRA Cell 1 is PCell, NR Cell 1 is PSCell | EUTRA: System information Combination 1 NR: N/A |

Table 7.1.2.1.1-2: Preamble parameters

| Execution Condition | Multi-PDN / Multi-PDU Sessions Condition | Generic Procedure Parameters | Primary DRB used for Data testing |
| --- | --- | --- | --- |
| IF pc_NG_RAN_NR | FALSE | Connectivity(NR), Test loop function(On) One DRB | Default DRB of the first PDU session on NR Cell |
|  | TRUE | Connectivity(NR), Test loop function(On) N DRBs (N ≥ 2) |  |
| ELSE IF pc_EN_DC | FALSE | Connectivity(EN-DC), DC bearer(One MN Terminated MCG bearer and One SN terminated SCG bearer), Test loop function(On) | SN Terminated SCG bearer unless explicitly specified in test case |
|  | TRUE | Connectivity(EN-DC), DC bearer(Two MN Terminated MCG bearer and One SN terminated SCG bearer), Test loop function(On) |  |
| ELSE IF pc_NGEN_DC | FALSE | Connectivity(NGEN-DC), DC bearer(One MN Terminated MCG bearer and One SN terminated SCG bearer), Test loop function(On) | SN Terminated SCG bearer unless explicitly specified in test case |
|  | TRUE | Connectivity(EN-DC), DC bearer(Two MN Terminated MCG bearer and One SN terminated SCG bearer), Test loop function(On) |  |

Table 7.1.2.1.1-3: Message conditions

| Execution Condition | Message condition exceptions |
| --- | --- |
| IF pc_NG_RAN_NR | Message with condition AM is used for step 7 in 4.5.4.2 according to [4] |
| ELSE IF pc_EN_DC | Message condition MCG_and_SCG with condition SCG-DRB(1,0) is used for step 7 in 4.5.4.2 according to [4] |
| ELSE IF pc_NGEN_DC | Message condition MCG_and_SCG with condition SCG-DRB(1,0) is used for step 7 in 4.5.4.2 according to [4] |

##### 7.1.2.1.2	Default Pre-Test Conditions for UM RLC test cases

Same Pre-test conditions as in clause 7.1.2.1.1 with the exceptions in Table 7.1.2.1.2-1.

Table 7.1.2.1.2-1: Message conditions

| Execution Condition | Message condition exceptions |
| --- | --- |
| IF pc_NG_RAN_NR | Message with condition UM is used for step 7 in 4.5.4.2 according to [4] |
| ELSE IF pc_EN_DC | Message condition MCG_and_SCG with condition SCG-DRB(0,1) is used for step 7 in 4.5.4.2 according to [4] |
| ELSE IF pc_NGEN_DC | Message condition MCG_and_SCG with condition SCG-DRB(0,1) is used for step 7 in 4.5.4.2 according to [4] |

#### 7.1.2.2	RLC Unacknowledged mode

##### 7.1.2.2.1	UM RLC / Segmentation and reassembly / 6-bit SN / Segmentation Info (SI) field

7.1.2.2.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state configured for 6 bit SN in RLC UM }

ensure that {

when { UE receives UMD PDU containing a SI field set to 00 }

then { UE correctly decodes the received UMD PDU }

}

(2)

with { UE in RRC_CONNECTED state configured for 6 bit SN in RLC UM }

ensure that {

when { UE receives a 6 bit SN configured UMD PDU containing a SI field set to 01 }

then { UE correctly decodes the received UMD PDU }

}

(3)

with { UE in RRC_CONNECTED state configured for 6 bit SN in RLC UM }

ensure that {

when { UE receives a 6 bit SN configured UMD PDU containing a SI field set to 11 and SO field }

then { UE correctly decodes the received UMD PDU }

}

(4)

with { UE in RRC_CONNECTED state configured for 6 bit SN in RLC UM }

ensure that {

when { UE receives a 6 bit SN configured UMD PDU containing a SI field set to 10 and SO field }

then { UE correctly decodes the received UMD PDU }

}

(5)

with { UE in RRC_CONNECTED state configured for 6 bit SN in RLC UM }

ensure that {

when { UE has UL SDU to send and UL grant available is sufficient to send whole SDU in one PDU }

then { UE transmits RLC SDU containing a SI field set to 00 }

}

(6)

with { UE in RRC_CONNECTED state configured for 6 bit SN in RLC UM }

ensure that {

when { UE has UL SDU to send and UL grant available is not sufficient to send whole SDU in one PDU }

then { UE transmits first RLC SDU segment containing a SI field set to 01 and including 6 bit SN }

}

(7)

with { UE in RRC_CONNECTED state configured for 6 bit SN in RLC UM }

ensure that {

when { UE has UL SDU to send and UL grant available is not sufficient to send whole SDU in one PDU }

then { UE transmits middle RLC SDU segment containing a SI field set to 11, including SO field and including 6 bit SN }

}

(8)

with { UE in RRC_CONNECTED state configured for 6 bit SN in RLC UM }

ensure that {

when { UE has UL SDU to send and UL grant available is not sufficient to send whole SDU in one PDU }

then { UE transmits last RLC SDU segment containing a SI field set to 10, including SO field and including 6 bit SN }

}

7.1.2.2.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.322, clauses 5.2.2.2.1, 5.2.2.2.2, 6.2.3.4 and 6.2.2.3. Unless otherwise stated these are Rel-15 requirements.

[TS 38.322, clause 5.2.2.2.1]

The receiving UM RLC entity shall maintain a reassembly window according to state variable RX_Next_Highest as follows:

-	a SN falls within the reassembly window if (RX_Next_Highest – UM_Window_Size) <= SN <RX_Next_Highest;

-	a SN falls outside of the reassembly window otherwise.

When receiving an UMD PDU from lower layer, the receiving UM RLC entity shall:

-	either deliver the UMD PDU after removing the RLC header, discard the received UMD PDU, or place it in the reception buffer (see sub clause 5.2.2.2.2);

-	if the received UMD PDU was placed in the reception buffer:

-	update state variables, reassemble and deliver RLC SDUs to upper layer and start/stop t-Reassembly as needed (see sub clause 5.2.2.2.3).

When t-Reassembly expires, the receiving UM RLC entity shall:

-	update state variables, discard RLC SDU segments and start t-Reassembly as needed (see sub clause 5.2.2.2.4).

[TS 38.322, clause 5.2.2.2.2]

When an UMD PDU is received from lower layer, the receiving UM RLC entity shall:

-	if the UMD PDU header does not contain an SN:

-	remove the RLC header and deliver the RLC SDU to upper layer.

-	else if (RX_Next_Highest – UM_Window_Size) <= SN < RX_Next_Reassembly:

-	discard the received UMD PDU.

-	else:

-	place the received UMD PDU in the reception buffer.

[TS 38.322, clause 6.2.2.3]

UMD PDU consists of a Data field and an UMD PDU header. The UMD PDU header is byte aligned

When an UMD PDU contains a complete RLC SDU, the UMD PDU header only contains the SI and R fields.

An UM RLC entity is configured by RRC to use either a 6 bit SN or a 12 bit SN. An UMD PDU header contains the SN field only when the corresponding RLC SDU is segmented. An UMD PDU carrying the first segment of an RLC SDU does not carry the SO field in its header. The length of the SO field is 16 bits.

Figure 6.2.2.3-1: UMD PDU containing a complete RLC SDU

Figure 6.2.2.3-2: UMD PDU with 6 bit SN (No SO)

Figure 6.2.2.3-3: UMD PDU with 12 bit SN (No SO)

Figure 6.2.2.3-4: UMD PDU with 6 bit SN and with SO

Figure 6.2.2.3-5: UMD PDU with 12 bit SN and with SO

[TS 38.322, clause 6.2.3.4]

Length: 2 bits.

The SI field indicates whether a RLC PDU contains a complete RLC SDU or the first, middle, last segment of a RLC SDU.

Table 6.2.2.6-1: SI field interpretation

| Value | Description |
| --- | --- |
| 00 | Data field contains all bytes of a RLC SDU |
| 01 | Data field contains the first segment of a RLC SDU |
| 10 | Data field contains the last segment of a RLC SDU |
| 11 | Data field contains neither the first nor last segment of a RLC SDU |

7.1.2.2.1.3	Test description

7.1.2.2.1.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.2.1.2 with the exception for the UM DRB is configured according to Table 7.1.2.2.1.3.1-1.

Table 7.1.2.2.1.3.1-1: RLC parameters

| Uplink UM RLC sn-FieldLength | size6 |
| --- | --- |
| Downlink UM RLC sn-FieldLength | size6 |

7.1.2.2.1.3.2	Test procedure sequence

Table 7.1.2.2.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | The SS stops allocating any UL grant. | - | - | - | - |
| 1 | The SS transmits UMD PDU#1 containing a complete RLC SDU#1 (SI field = 00). | <-- | UMD PDU#1 | - | - |
| 2 | SS allocates an UL grant sufficient to loop back RLC SDU#1 in one RLC/MAC PDU | <-- | UL Grant | - | - |
| 3 | Check: Does the UE transmit RLC SDU#1? | --> | (RLC SDU#1) | 1,5 | P |
| 4 | The SS transmits UMD PDU#2 containing the first segment of RLC SDU#2 (SI field = 01). Note 3 | <-- | UMD PDU#2 | - | - |
| 5 | The SS transmits UMD PDU#3 containing the second segment of RLC SDU#2 (SI field = 11) and including SO field. Note 3 | <-- | UMD PDU#3 | - | - |
| 6 | The SS transmits UMD PDU#4 containing the last segment of RLC SDU#2 (SI field = 10) and including SO field. Note 3 | <-- | UMD PDU#4 | - | - |
| 7 | SS allocates 3 UL grants at an interval of 20 ms so as to loop back RLC SDU#2 in 3 RLC/MAC PDUs. Note 1 & 2 | <-- | UL Grants | - | - |
| 8 | Check: Does the UE transmit UMD PDU#2 containing the first segment of RLC SDU#2 (SI field = 01)? | --> | (RLC SDU#2, first segment) | 2,3,4,6 | P |
| 9 | Check: Does the UE transmit UMD PDU#3 containing the second segment of RLC SDU#2 (SI field = 11) and including SO field? | --> | (RLC SDU#2, second segment) | 2,3,4,7 | P |
| 10 | Check: Does the UE transmit UMD PDU#4 containing the last segment of RLC SDU#2 (SI field = 10) and including SO field? | --> | (RLC SDU#2, last segment) | 2,3,4,8 | P |
| Note 1: The UL grants for step 8,9,10 are sufficiently small (240 bits, LRBs & IMCS as per 38.523-3[3] annex B) that UE transmits RLC SDU#2 in 3 UL RLC PDUs by segmenting. Note 2: The RLC PDU containing a segment shall be of size 208 bits resp. 224 bits and a MAC sub PDU header of 16 bits and a 16-bit MAC BSR CE included in step 8 resulting in a MAC PDU of size 240 bits. Note 3: The data part in step 4 first segment not including SO is 200 bits (25 bytes). Step 5, second segment SO=25 and data is 200 bits (25 bytes). Step 6, third segment SO=25+25=50 and data is 200 bits (25 bytes). |  |  |  |  |  |

7.1.2.2.1.3.3	Specific message contents

None.

##### 7.1.2.2.2	UM RLC / Segmentation and reassembly / 12-bit SN / Segmentation Info (SI) field

7.1.2.2.2.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state configured for 12 bit SN in RLC UM }

ensure that {

when { UE receives UMD PDU containing a SI field set to 00 }

then { UE correctly decodes the received UMD PDU }

}

(2)

with { UE in RRC_CONNECTED state configured for 12 bit SN in RLC UM }

ensure that {

when { UE receives a 12 bit SN configured UMD PDU containing a SI field set to 01 }

then { UE correctly decodes the received UMD PDU }

}

(3)

with { UE in RRC_CONNECTED state configured for 12 bit SN in RLC UM }

ensure that {

when { UE receives a 12 bit SN configured UMD PDU containing a SI field set to 11 and SO field }

then { UE correctly decodes the received UMD PDU }

}

(4)

with { UE in RRC_CONNECTED state configured for 12 bit SN in RLC UM }

ensure that {

when { UE receives a 12 bit SN configured UMD PDU containing a SI field set to 10 and SO field }

then { UE correctly decodes the received UMD PDU }

}

(5)

with { UE in RRC_CONNECTED state configured for 12 bit SN in RLC UM }

ensure that {

when { UE has UL SDU to send and UL grant available is sufficient to send whole SDU in one PDU }

then { UE transmits RLC SDU containing a SI field set to 00  }

}

(6)

with { UE in RRC_CONNECTED state configured for 12 bit SN in RLC UM }

ensure that {

when { UE has UL SDU to send and UL grant available is not sufficient to send whole SDU in one PDU }

then { UE transmits first RLC SDU segment containing a SI field set to 01 and including 12 bit SN}

}

(7)

with { UE in RRC_CONNECTED state configured for 12 bit SN in RLC UM }

ensure that {

when { UE has UL SDU to send and UL grant available is not sufficient to send whole SDU in one PDU }

then { UE transmits middle RLC SDU segment containing a SI field set to 11, including SO field and including 12 bit SN }

}

(8)

with { UE in RRC_CONNECTED state configured for 12 bit SN in RLC UM }

ensure that {

when { UE has UL SDU to send and UL grant available is not sufficient to send whole SDU in one PDU }

then { UE transmits last RLC SDU segment containing a SI field set to 10, including SO field and including 12 bit SN }

}

7.1.2.2.2.2	Conformance requirements

Same conformance requirements as clause 7.1.2.2.1.2

7.1.2.2.2.3	Test description

7.1.2.2.2.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.2.1.2 with the exception for the UM DRB is configured according to Table 7.1.2.2.2.3.1-1.

Table 7.1.2.2.2.3.1-1: RLC parameters

| Uplink UM RLC sn-FieldLength | size12 |
| --- | --- |
| Downlink UM RLC sn-FieldLength | size12 |

7.1.2.2.2.3.2	Test procedure sequence

Same test procedure sequence as 7.1.2.2.1.3.2 except that RLC UM SN is 12 bit and the data part in step 4 first segment not including SO is 192 bits (24 Bytes). Step 5, second segment SO=24 and data is 192 bits (24 bytes). Step 6, third segment SO=24+24=48 and data is 192 bits (24 bytes).

7.1.2.2.2.3.3	Specific message contents

None.

##### 7.1.2.2.3	UM RLC / 6-bit SN / Correct use of sequence numbering

7.1.2.2.3.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with UM RLC 6 bit SN }

ensure that {

when { UE transmits the first PDU which is segmented }

then { UE includes the SN field equal to 0 in each RLC segment }

}

(2)

with { UE in RRC_CONNECTED state with UM RLC 6 bit SN }

ensure that {

when { UE transmit subsequent segmented PDUs }

then { UE includes the SN field incremented by 1 for each segmented PDU of one RLC SDU }

}

(3)

with { UE in RRC_CONNECTED state with UM RLC 6 bit SN }

ensure that {

when { UE transmit segments belonging to more than 64 SDUs }

then { UE wraps the SN after transmitting the segments of 64 SDUs }

}

(4)

with { UE in RRC_CONNECTED state with UM RLC 6 bit SN }

ensure that {

when { segments of more than 64 SDUs are sent to UE }

then { UE accepts PDUs with SNs that wrap around every 64 segmented SDUs }

}

7.1.2.2.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 38.322, clause 5.2.2.1.1, 5.2.2.2, 6.2.2.3, 6.2.3.3 and 7.1. Unless otherwise stated these are Rel-15 requirements.

[TS 38.322, clause 5.2.2.1.1]

When submitting a UMD PDU to lower layer, the transmitting UM RLC entity shall:

-	if the UMD PDU contains a segment of an RLC SDU, set the SN of the UMD PDU to TX_Next;

-	if the UMD PDU contains a segment that maps to the last byte of an RLC SDU, then increment TX_Next by one.

[TS 38.322, clause 5.2.2.2]

The receiving UM RLC entity shall maintain a reassembly window according to state variable RX_Next_Highest as follows:

-	a SN falls within the reassembly window if (RX_Next_Highest – UM_Window_Size) <= SN <RX_Next_Highest;

-	a SN falls outside of the reassembly window otherwise.

When receiving an UMD PDU from lower layer, the receiving UM RLC entity shall:

-	either deliver the UMD PDU after removing the RLC header, discard the received UMD PDU, or place it in the reception buffer (see sub clause 5.2.2.2.2);

-	if the received UMD PDU was placed in the reception buffer:

-	update state variables, reassemble and deliver RLC SDUs to upper layer and start/stop t-Reassembly as needed (see sub clause 5.2.2.2.3).

…

When an UMD PDU is received from lower layer, the receiving UM RLC entity shall:

-	if the UMD PDU header does not contain an SN:

-	remove the RLC header and deliver the RLC SDU to upper layer.

-	else if (RX_Next_Highest – UM_Window_Size) <= SN < RX_Next_Reassembly:

-	discard the received UMD PDU.

-	else:

-	place the received UMD PDU in the reception buffer.

…

When an UMD PDU with SN = x is placed in the reception buffer, the receiving UM RLC entity shall:

-	if all byte segments with SN = x are received:

-	reassemble the RLC SDU from all byte segments with SN = x, remove RLC headers and deliver the reassembled RLC SDU to upper layer;

-	if x = RX_Next_Reassembly:

-	update RX_Next_Reassembly to the SN of the first SN > current RX_Next_Reassembly that has not been reassembled and delivered to upper layer.

-	else if x falls outside of the reassembly window:

-	update RX_Next_Highest to x + 1;

-	discard any UMD PDUs with SN that falls outside of the reassembly window;

-	if RX_Next_Reassembly falls outside of the reassembly window:

-	set RX_Next_Reassembly to the SN of the first SN >= (RX_Next_Highest – UM_Window_Size) that has not been reassembled and delivered to upper layer.

[TS 38.322, clause 6.2.2.3]

An UM RLC entity is configured by RRC to use either a 6 bit SN or a 12 bit SN. An UMD PDU header contains the SN field only when the corresponding RLC SDU is segmented.

[TS 38.322, clause 6.2.3.3]

The SN field indicates the sequence number of the corresponding RLC SDU. …. For RLC UM, the sequence number is incremented by one for every segmented RLC SDU..

[TS 38.322, clause 7.1]

All state variables and all counters are non-negative integers.

…

All state variables related to UM data transfer can take values from 0 to 63 for 6 bit SN or from 0 to 4095 for 12 bit SN. All arithmetic operations contained in the present document on state variables related to UM data transfer are affected by the UM modulus (i.e. final value = [value from arithmetic operation] modulo 64 for 6 bit SN and 4096 for 12 bit SN).

…

Each transmitting UM RLC entity shall maintain the following state variables:

a) TX_Next

This state variable holds the value of the SN to be assigned for the next newly generated UMD PDU with segment. It is initially set to 0, and is updated after the UM RLC entity submits a UMD PDU including the last segment of an RLC SDU to lower layers.

Each receiving UM RLC entity shall maintain the following state variables and constant:

b) RX_Next_Reassembly – UM receive state variable

This state variable holds the value of the earliest SN that is still considered for reassembly. It is initially set to 0.

c) RX_Timer_Trigger – UM t-Reassembly state variable

This state variable holds the value of the SN following the SN which triggered t-Reassembly.

d) RX_Next_Highest– UM receive state variable

This state variable holds the value of the SN following the SN of the UMD PDU with the highest SN among received UMD PDUs. It serves as the higher edge of the reassembly window. It is initially set to 0.

7.1.2.2.3.3	Test description

7.1.2.2.3.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.2.1.2 with the exception for the UM DRB is configured according to Table 7.1.2.2.3.3.1-1.

Table 7.1.2.2.3.3.1-1: RLC parameters

| Uplink UM RLC sn-FieldLength | size6 |
| --- | --- |
| Downlink UM RLC sn-FieldLength | size6 |

7.1.2.2.3.3.2	Test procedure sequence

Table 7.1.2.2.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | The SS stops allocating any UL grant. | - | - | - | - |
| 1 | The SS transmits UMD PDU#1 with 6 bit SN = 0 containing the first segment of RLC SDU#1 (SI field = 01). | <-- | UMD PDU#1 | - | - |
| 2 | The SS transmits UMD PDU#2 with 6 bit SN=0 containing the last segment of RLC SDU#1 (SI field = 10) and including SO field. | <-- | UMD PDU#2 | - | - |
| 3 | SS allocates 2 UL grants at an interval of 20 ms so as to loop back RLC SDU#1 in 2 RLC/MAC PDUs. (Note 1) | <-- | UL Grants | - | - |
| 4 | Check: Does the UE transmit UMD PDU#1 with 6 bit SN = 0 containing the first segment of RLC SDU#1 (SI field = 01)? | --> | (RLC SDU#1, first segment) | 1 | P |
| 5 | Check: Does the UE transmit UMD PDU#2 with 6 bit SN = 0 containing the last segment of RLC SDU#1 (SI field = 10)? | --> | (RLC SDU#1, last segment) | 1 | P |
| - | EXCEPTION: Steps 6 to 10 are executed 63 times, the initial value of k = 1, it is incremented by one for each iteration. | - | - | - | - |
| 6 | The SS transmits UMD PDU#(2*k+1) with 6 bit SN = k containing the first segment of RLC SDU#(k+1) (SI field = 01). | <-- | UMD PDU#(2*k+1) | - | - |
| 7 | The SS transmits UMD PDU#(2*(k+1)) with 6 bit SN=k containing the last segment of RLC SDU#(k+1) (SI field = 10) | <-- | UMD PDU#(2*(k+1)) | - | - |
| 8 | SS allocates 2 UL grants at an interval of 20 ms so as to loop back RLC SDU#(k+1) in 2 RLC/MAC PDUs. (Note 1) | <-- | UL Grants | - | - |
| 9 | Check: Does the UE transmit UMD PDU#(2*k+1) with 6 bit SN = k containing the first segment of RLC SDU#(k+1) (SI field = 01)? (Note 2) | --> | (RLC SDU#(k+1), first segment) | 2 | P |
| 10 | Check: Does the UE transmit UMD PDU#(2*(k+1)) with 6 bit SN = k containing the last segment of RLC SDU#(k+1) (SI field = 10) and including SO field? (Note 2) | --> | (RLC SDU#(k+1), last segment) | 2 | P |
| 11 | The SS transmits UMD PDU#129 with 6 bit SN = 0 containing the first segment of RLC SDU#65 (SI field = 01). | <-- | UMD PDU#129 | - | - |
| 12 | The SS transmits UMD PDU#130 with 6 bit SN= 0 containing the last segment of RLC SDU#65 (SI field = 10) and including SO field | <-- | UMD PDU#130 | - | - |
| 13 | SS allocates 2 UL grants at an interval of 20 ms so as to loop back RLC SDU#65 in 2 RLC/MAC PDUs. (Note 1) | <-- | UL Grants | - | - |
| 14 | Check: Does the UE transmit UMD PDU#129 with 6 bit SN = 0 containing the first segment of RLC SDU#65 (SI field = 01)? | --> | (RLC SDU#65, first segment) | 3.4 | P |
| 15 | Check: Does the UE transmit UMD PDU#130 with 6 bit SN = 0 containing the last segment of RLC SDU#65 (SI field = 10) and including SO field? | --> | (RLC SDU#65, last segment) | 3,4 | P |
| Note 1: The RLC SDU size shall be 12 octets which are segmented into 6 and 6 octets. With 2 octets of MAC header, 2 octets of Short BSR and 1 octet of RLC header (without SO) the first segment consists of 88 bits and a TBS of this size shall be allocated. With 2 octets of MAC header and 3 octets of RLC header (with SO) the second segment consists of 88 bits and a TBS of this size shall be allocated. (LRBs & IMCS as per 38.523-3[3] annex B) Note 2: The verdict shall be provided each time (SN+1) mod 16 = 0. |  |  |  |  |  |

7.1.2.2.3.3.3	Specific message contents

None.

##### 7.1.2.2.4	UM RLC / 12-bit SN / Correct use of sequence numbering

7.1.2.2.4.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with UM RLC 12 bit SN }

ensure that {

when { UE transmits the first PDU which is segmented }

then { UE includes the SN field equal to 0 in each RLC segment }

}

(2)

with { UE in RRC_CONNECTED state with UM RLC 12 bit SN }

ensure that {

when { UE transmit subsequent segmented PDUs }

then { UE includes the SN field incremented by 1 for each segmented PDU of one RLC SDU}

}

(3)

with { UE in RRC_CONNECTED state with UM RLC 12 bit SN }

ensure that {

when { UE transmit segments belonging to more than 4096 SDUs }

then { UE wraps the SN after transmitting the segments of 4096 SDUs }

}

(4)

with { UE in RRC_CONNECTED state with UM RLC 12 bit SN }

ensure that {

when { segments of more than 4096 SDUs are sent to UE }

then { UE accepts PDUs with SNs that wrap around every 4096 segmented SDUs }

}

7.1.2.2.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 38.322, clause 5.2.2.1.1, 5.2.2.2, 6.2.2.3, 6.2.3.3 and 7.1. Unless otherwise stated these are Rel-15 requirements.

[TS 38.322, clause 5.2.2.1.1]

When submitting a UMD PDU to lower layer, the transmitting UM RLC entity shall:

-	if the UMD PDU contains a segment of an RLC SDU, set the SN of the UMD PDU to TX_Next;

-	if the UMD PDU contains a segment that maps to the last byte of an RLC SDU, then increment TX_Next by one.

[TS 38.322, clause 5.2.2.2]

The receiving UM RLC entity shall maintain a reassembly window according to state variable RX_Next_Highest as follows:

-	a SN falls within the reassembly window if (RX_Next_Highest – UM_Window_Size) <= SN <RX_Next_Highest;

-	a SN falls outside of the reassembly window otherwise.

When receiving an UMD PDU from lower layer, the receiving UM RLC entity shall:

-	either deliver the UMD PDU after removing the RLC header, discard the received UMD PDU, or place it in the reception buffer (see sub clause 5.2.2.2.2);

-	if the received UMD PDU was placed in the reception buffer:

-	update state variables, reassemble and deliver RLC SDUs to upper layer and start/stop t-Reassembly as needed (see sub clause 5.2.2.2.3).

…

When an UMD PDU is received from lower layer, the receiving UM RLC entity shall:

-	if the UMD PDU header does not contain an SN:

-	remove the RLC header and deliver the RLC SDU to upper layer.

-	else if (RX_Next_Highest – UM_Window_Size) <= SN < RX_Next_Reassembly:

-	discard the received UMD PDU.

-	else:

-	place the received UMD PDU in the reception buffer.

…

When an UMD PDU with SN = x is placed in the reception buffer, the receiving UM RLC entity shall:

-	if all byte segments with SN = x are received:

-	reassemble the RLC SDU from all byte segments with SN = x, remove RLC headers and deliver the reassembled RLC SDU to upper layer;

-	if x = RX_Next_Reassembly:

-	update RX_Next_Reassembly to the SN of the first SN > current RX_Next_Reassembly that has not been reassembled and delivered to upper layer.

-	else if x falls outside of the reassembly window:

-	update RX_Next_Highest to x + 1;

-	discard any UMD PDUs with SN that falls outside of the reassembly window;

-	if RX_Next_Reassembly falls outside of the reassembly window:

-	set RX_Next_Reassembly to the SN of the first SN >= (RX_Next_Highest – UM_Window_Size) that has not been reassembled and delivered to upper layer.

[TS 38.322, clause 6.2.2.3]

An UM RLC entity is configured by RRC to use either a 6 bit SN or a 12 bit SN. An UMD PDU header contains the SN field only when the corresponding RLC SDU is segmented.

[TS 38.322, clause 6.2.3.3]

The SN field indicates the sequence number of the corresponding RLC SDU. … For RLC UM, the sequence number is incremented by one for every segmented RLC SDU.

[TS 38.322, clause 7.1]

All state variables and all counters are non-negative integers.

…

All state variables related to UM data transfer can take values from 0 to 63 for 6 bit SN or from 0 to 4095 for 12 bit SN. All arithmetic operations contained in the present document on state variables related to UM data transfer are affected by the UM modulus (i.e. final value = [value from arithmetic operation] modulo 64 for 6 bit SN and 4096 for 12 bit SN).

…

Each transmitting UM RLC entity shall maintain the following state variables:

a) TX_Next

This state variable holds the value of the SN to be assigned for the next newly generated UMD PDU with segment. It is initially set to 0, and is updated after the UM RLC entity submits a UMD PDU including the last segment of an RLC SDU to lower layers.

Each receiving UM RLC entity shall maintain the following state variables and constant:

b) RX_Next_Reassembly – UM receive state variable

This state variable holds the value of the earliest SN that is still considered for reassembly. It is initially set to 0.

c) RX_Timer_Trigger – UM t-Reassembly state variable

This state variable holds the value of the SN following the SN which triggered t-Reassembly.

d) RX_Next_Highest– UM receive state variable

This state variable holds the value of the SN following the SN of the UMD PDU with the highest SN among received UMD PDUs. It serves as the higher edge of the reassembly window. It is initially set to 0.

7.1.2.2.4.3	Test description

7.1.2.2.4.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.2.1.2 with the exception for the UM DRB is configured according to Table 7.1.2.2.4.3.1-1.

Table 7.1.2.2.4.3.1-1: RLC parameters

| Uplink RLC sn-FieldLength | size12 |
| --- | --- |
| Downlink RLC sn-FieldLength | size12 |

7.1.2.2.4.3.2	Test procedure sequence

Table 7.1.2.2.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits UMD PDU#1 with 12 bit SN = 0 containing the first segment of RLC SDU#1 (SI field = 01). | <-- | UMD PDU#1 | - | - |
| 2 | The SS transmits UMD PDU#2 with 12 bit SN=0 containing the last segment of RLC SDU#1 (SI field = 10) and including SO field | <-- | UMD PDU#2 | - | - |
| 3 | SS allocates 2 UL grants at an interval of 20 ms so as to loop back RLC SDU#1 in 2 RLC/MAC PDUs. (Note 1) | <-- | UL Grants | - | - |
| 4 | Check: Does the UE transmit UMD PDU#1 with 12 bit SN = 0 containing the first segment of RLC SDU#1 (SI field = 01)? | --> | (RLC SDU#1, first segment) | 1 | P |
| 5 | Check: Does the UE transmit UMD PDU#2 with 12 bit SN = 0 containing the last segment of RLC SDU#1 (SI field = 10)? | --> | (RLC SDU#1, last segment) | 1 | P |
| - | EXCEPTION: Steps 6 to 10 are executed 4095 times, the initial value of k = 1, it is incremented by one for each iteration. | - | - | - | - |
| 6 | The SS transmits UMD PDU#(2*k+1) with 12 bit SN = k containing the first segment of RLC SDU#(k+1) (SI field = 01). | <-- | UMD PDU#(2*k+1) | - | - |
| 7 | The SS transmits UMD PDU#(2*(k+1)) with 12 bit SN=k containing the last segment of RLC SDU#(k+1) (SI field = 10) | <-- | UMD PDU#(2*(k+1)) | - | - |
| 8 | SS allocates 2 UL grants at an interval of 20 ms so as to loop back RLC SDU#(k+1) in 2 RLC/MAC PDUs. (Note 1) | <-- | UL Grants | - | - |
| 9 | Check: Does the UE transmit UMD PDU#(2*k+1) with 12 bit SN = k containing the first segment of RLC SDU#(k+1) (SI field = 01)? (Note 2) | --> | (RLC SDU#(k+1), first segment) | 2 | P |
| 10 | Check: Does the UE transmit UMD PDU#(2*(k+1)) with 12 bit SN = k containing the last segment of RLC SDU#(k+1) (SI field = 10) and including SO field? (Note 2) | --> | (RLC SDU#(k+1), last segment) | 2 | P |
| 11 | The SS transmits UMD PDU#8193 with 12 bit SN = 0 containing the first segment of RLC SDU#4097 (SI field = 01). | <-- | UMD PDU#8193 | - | - |
| 12 | The SS transmits UMD PDU#8194 with 12 bit SN= 0 containing the last segment of RLC SDU#4097 (SI field = 10) and including SO field | <-- | UMD PDU#8194 | - | - |
| 13 | SS allocates 2 UL grants at an interval of 20 ms so as to loop back RLC SDU#4097 in 2 RLC/MAC PDUs. (Note 1) | <-- | UL Grants | - | - |
| 14 | Check: Does the UE transmit UMD PDU#8193 with 12 bit SN = 0 containing the first segment of RLC SDU#4097 (SI field = 01)? | --> | (RLC SDU#4097, first segment) | 3.4 | P |
| 15 | Check: Does the UE transmit UMD PDU#8194 with 12 bit SN = 0 containing the last segment of RLC SDU#4097 (SI field = 10) and including SO field? | --> | (RLC SDU#4097, last segment) | 3,4 | P |
| Note 1: The RLC SDU size shall be 10 octets which are segmented into 5 and 5 octets. With 2 octets of MAC header, 2 octets of Short BSR and 2 octets of RLC header (without SO) the first segment consists of 88 bits and a TBS of this size shall be allocated. With 2 octets of MAC header and 4 octets of RLC header (with SO) the second segment consists of 88 bits and a TBS of this size shall be allocated. (LRBs & IMCS as per 38.523-3[3] annex B) Note 2: The verdict shall be provided each time (SN+1) mod 256 = 0. |  |  |  |  |  |

7.1.2.2.4.3.3	Specific message contents

None.

##### 7.1.2.2.5	UM RLC / Receive Window operation and t-Reassembly expiry

7.1.2.2.5.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and using UM RLC }

ensure that {

when { UE receives a RLC PDU including SN and '(RX_Next_Highest – UM_Window_Size) <= SN < RX_Next_Highest }

then { UE discards any UMD PDUs with SN that falls outside of the reassembly window }

}

(2)

with { UE in RRC_CONNECTED state and using UM RLC }

ensure that {

when { UE receives a RLC PDU including SN and '(RX_Next_Highest – UM_Window_Size) > SN or SN >= RX_Next_Reassembly' }

then { UE stores the PDU in receive buffer }

}

(3)

with { UE in RRC_CONNECTED state and using UM RLC }

ensure that {

when { UE places a RLC PDU including SN into the reception buffer and all byte segments with that SN are received }

then { UE delivers the reassembled SDU to upper layers }

}

(4)

with { UE in RRC_CONNECTED state and using UM RLC }

ensure that {

when { t-Reassembly expires }

then { UE updates RX_Next_Reassembly and discards all segments with SN < updated RX_Next_Reassembly }

}

7.1.2.2.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.322, clauses 5.2.2.2.1, 5.2.2.2.2, 5.2.2.2.3, 5.2.2.2.4 and 7.1. Unless otherwise stated these are Rel-15 requirements.

[TS 38.322, clause 5.2.2.2.1]

The receiving UM RLC entity shall maintain a reassembly window according to state variable RX_Next_Highest as follows:

-	a SN falls within the reassembly window if (RX_Next_Highest – UM_Window_Size) <= SN <RX_Next_Highest;

-	a SN falls outside of the reassembly window otherwise.

When receiving an UMD PDU from lower layer, the receiving UM RLC entity shall:

-	either deliver the UMD PDU to upper layer after removing the RLC header, discard the received UMD PDU, or place it in the reception buffer (see sub clause 5.2.2.2.2);

-	if the received UMD PDU was placed in the reception buffer:

-	update state variables, reassemble and deliver RLC SDUs to upper layer and start/stop t-Reassembly as needed (see sub clause 5.2.2.2.3).

When t-Reassembly expires, the receiving UM RLC entity shall:

-	update state variables, discard RLC SDU segments and start t-Reassembly as needed (see sub clause 5.2.2.2.4).

[TS 38.322, clause 5.2.2.2.2]

When an UMD PDU is received from lower layer, the receiving UM RLC entity shall:

-	if the UMD PDU header does not contain an SN:

-	remove the RLC header and deliver the RLC SDU to upper layer.

-	else if (RX_Next_Highest – UM_Window_Size) <= SN < RX_Next_Reassembly:

-	discard the received UMD PDU.

-	else:

-	place the received UMD PDU in the reception buffer.

[TS 38.322, clause 5.2.2.2.3]

When an UMD PDU with SN = x is placed in the reception buffer, the receiving UM RLC entity shall:

-	if all byte segments with SN = x are received:

-	reassemble the RLC SDU from all byte segments with SN = x, remove RLC headers and deliver the reassembled RLC SDU to upper layer;

-	if x = RX_Next_Reassembly:

-	update RX_Next_Reassembly to the SN of the first SN > current RX_Next_Reassembly that has not been reassembled and delivered to upper layer.

-	else if x falls outside of the reassembly window:

-	update RX_Next_Highest to x + 1;

-	discard any UMD PDUs with SN that falls outside of the reassembly window;

-	if RX_Next_Reassembly falls outside of the reassembly window:

-	set RX_Next_Reassembly to the SN of the first SN >= (RX_Next_Highest – UM_Window_Size) that has not been reassembled and delivered to upper layer.

-	if t-Reassembly is running:

-	if RX_Timer_Trigger <= RX_Next_Reassembly; or

-	if RX_Timer_Trigger falls outside of the reassembly window and RX_Timer_Trigger is not equal to RX_Next_Highest; or

-	if RX_Next_Highest = RX_Next_Reassembly + 1 and there is no missing byte segment of the RLC SDU associated with SN = RX_Next_Reassembly before the last byte of all received segments of this RLC SDU:

-	stop and reset t-Reassembly.

-	if t-Reassembly is not running (includes the case when t-Reassembly is stopped due to actions above):

-	if RX_Next_Highest > RX_Next_Reassembly + 1; or

-	if RX_Next_Highest = RX_Next_Reassembly + 1 and there is at least one missing byte segment of the RLC SDU associated with SN = RX_Next_Reassembly before the last byte of all received segments of this RLC SDU:

-	start t-Reassembly;

-	set RX_Timer_Trigger to RX_Next_Highest.

[TS 38.322, clause 5.2.2.2.4]

When t-Reassembly expires, the receiving UM RLC entity shall:

-	update RX_Next_Reassembly to the SN of the first SN >= RX_Timer_Trigger that has not been reassembled;

-	discard all segments with SN < updated RX_Next_Reassembly;

-	if RX_Next_Highest > RX_Next_Reassembly + 1; or

-	if RX_Next_Highest = RX_Next_Reassembly + 1 and there is at least one missing byte segment of the RLC SDU associated with SN = RX_Next_Reassembly before the last byte of all received segments of this RLC SDU:

-	start t-Reassembly;

-	set RX_Timer_Trigger to RX_Next_Highest.

[TS 38.322, clause 5.2.2.2.4]

This sub clause describes the state variables used in AM and UM entities in order to specify the RLC protocol. The state variables defined in this subclause are normative.

All state variables and all counters are non-negative integers.

...

All state variables related to UM data transfer can take values from 0 to 63 for 6 bit SN or from 0 to 4095 for 12 bit SN. All arithmetic operations contained in the present document on state variables related to UM data transfer are affected by the UM modulus (i.e. final value = [value from arithmetic operation] modulo 64 for 6 bit SN and 4096 for 12 bit SN).

When performing arithmetic comparisons of state variables or SN values, a modulus base shall be used.

...

RX_Next_Highest– UM_Window_Size shall be assumed as the modulus base at the receiving side of an UM RLC entity. This modulus base is subtracted from all the values involved, and then an absolute comparison is performed (e.g. (RX_Next_Highest– UM_Window_Size) <= SN < RX_Next_Highest is evaluated as [(RX_Next_Highest– UM_Window_Size) – (RX_Next_Highest– UM_Window_Size)] modulo 2[sn-FieldLength] <= [SN – (RX_Next_Highest– UM_Window_Size)] modulo 2[sn-FieldLength] < [RX_Next_Highest– (RX_Next_Highest– UM_Window_Size)] modulo 2[sn-FieldLength]), where sn-FieldLength is 6 or 12 for 6 bit SN and 12 bit SN, respectively.

...

Each transmitting UM RLC entity shall maintain the following state variables:

a) TX_Next

This state variable holds the value of the SN to be assigned for the next newly generated UMD PDU with segment. It is initially set to 0, and is updated after the UM RLC entity submits a UMD PDU including the last segment of an RLC SDU to lower layers.

Each receiving UM RLC entity shall maintain the following state variables and constant:

b) RX_Next_Reassembly – UM receive state variable

This state variable holds the value of the earliest SN that is still considered for reassembly. It is initially set to 0.

c) RX_Timer_Trigger – UM t-Reassembly state variable

This state variable holds the value of the SN following the SN which triggered t-Reassembly.

d) RX_Next_Highest– UM receive state variable

This state variable holds the value of the SN following the SN of the UMD PDU with the highest SN among received UMD PDUs. It serves as the higher edge of the reassembly window. It is initially set to 0.

7.1.2.2.5.3	Test description

7.1.2.2.5.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.2.1.2 with the exception that the UM DRB is configured according to Table 7.1.2.2.5.3.1-1.

Table 7.1.2.2.5.3.1-1: RLC parameters

| t-Reassembly | ms200 |
| --- | --- |
| Uplink UM RLC sn-FieldLength | IF (pc_um_WithShortSN ) size6 ELSE size12 |
| Downlink UM RLC sn-FieldLength | F (pc_um_WithShortSN ) size6 ELSE size12 |

Table 7.1.2.2.5.3.1-2: PDCP Settings

| Parameter | Value |
| --- | --- |
| t-Reordering | ms30 |

7.1.2.2.5.3.2	Test procedure sequence

Table 7.1.2.2.5.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | The SS ignores scheduling requests and does not allocate any uplink grant. | - | - | - | - |
| 1 | The SS transmits UMD PDU#1 containing first segment of RLC SDU#1, SN=0. | <-- | UMD PDU#1 | - | - |
| 2 | 20 ms after step 1 the SS transmits UMD PDU#3 containing first segment of RLC SDU#2, SN=1. | <-- | UMD PDU#3 | - | - |
| 3 | 40 ms after step 1 the SS transmits UMD PDU#4 containing last segment of RLC SDU#2, SN=1. | <-- | UMD PDU#4 | - | - |
| 3A | 60 ms after step 1 the SS transmits UMD PDU#9 containing first segment of RLC SDU#5, SN=w | <-- | UMD PDU#9 | - | - |
| 3B | 80 ms after step 1 the SS transmits UMD PDU#10 containing last segment of RLC SDU#5, SN=w | <-- | UMD PDU#10 | - | - |
| 3C | 100 ms after step 1 the SS assigns 2 UL grants (UL grant allocation type 2) with a time spacing of 20 ms so as to loop back RLC SDU#2. | - | - | - | - |
| 4 | Check: Does the UE transmit RLC SDU#2? (Note 3) | --> | (RLC SDU#2) | 2,3 | P |
| 4A | Check: Does the UE transmit RLC SDU#5? (Note 4) | --> | (RLC SDU#5) | 2,3 | P |
| 5 | 160 ms after step 1 the SS transmits UMD PDU#2 last segment of RLC SDU#1, SN=0. | <-- | UMD PDU#2 | - | - |
| 5A | The SS starts the UL default grant transmissions. | - | - | - | - |
| 6 | Check: For 1 sec after step 5, does the UE transmit RLC SDU#1, SN=0? (Note 6) | --> | (RLC SDU#1) | 1 | F |
| 6A |  | - | - | - | - |
| 7 | The SS transmits UMD PDU#5 containing first segment of RLC SDU#3, SN=5. | <-- | UMD PDU#5 | - | - |
| 8 | Wait for 200 ms to ensure that t- Reassembly for the UMD PDU#5 expires. | - | - | - | - |
| 9 | The SS transmits UMD PDU#6 containing last segment of RLC SDU#3, SN=5 (Note 7). | <-- | UMD PDU#6 | - | - |
| 10 | Check: For 1 sec after step 9, does the UE transmit RLC SDU#3? (Note 7) | --> | (RLC SDU#3) | 4 | F |
| 11 | The SS transmits UMD PDU#7 containing first segment of RLC SDU#6, SN=8. | <-- | UMD PDU#7 | - | - |
| 12 | The SS transmits UMD PDU#8 containing last segment of RLC SDU#6, SN=8. | <-- | UMD PDU#8 | - | - |
| 13 | Check: Does the UE transmit RLC SDU#6? (Note 5) | --> | (RLC SDU#6) | 2,3 | P |
| Note 1: The RLC SDU size shall be 12 octets which are segmented into 7 and 5 octets. Note 2: UL grant of 144 bits(LRBs & IMCS as per 38.523-3[3] annex B) is chosen to allow the UE to transmit one PDU at a time( 12 bytes RLC SDU + 1 or 2 bytes RLC Header + 2 bytes MAC Sub PDU header + 2 or 3 bytes for short BSR and/or padding). Note 3: The UE transmits the looped back PDCP data of RLC SDU#2 in a PDCP PDU with PDCP SN=0. Note 4: The UE transmits the looped back PDCP data of RLC SDU#5 in a PDCP PDU with PDCP SN=1. Note 5: The UE transmits the looped back PDCP data of RLC SDU#6 in a PDCP PDU with PDCP SN=2. Note 6: The UE transmits the looped back PDCP data of RLC SDU#1 in a PDCP PDU with PDCP SN=2. Note 7: The UE transmits the looped back PDCP data of RLC SDU#3 in a PDCP PDU with PDCP SN=2. |  |  |  |  |  |

7.1.2.2.5.3.3	Specific message contents

None

##### 7.1.2.2.5a	NR NTN / UM RLC / t-Reassembly expiry / t-ReassemblyExt-r17 configured

7.1.2.2.5a.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state on a cell which provides access by NR NTN and using UM RLC }

ensure that {

when { t-ReassemblyExt-r17 is configured }

then { UE ignores value signalled in t-Reassembly and updates RX_Next_Reassembly and discards all segments with SN < updated RX_Next_Reassembly after the expiry of t-ReassemblyExt-r17 }

}

7.1.2.2.5a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.322, clauses 5.2.2.2.1, 5.2.2.2.3, 5.2.2.2.4, 7.1 and TS 38.331, clause 6.3.2. Unless otherwise stated these are Rel-17 requirements.

[TS 38.322, clause 5.2.2.2.1]

The receiving UM RLC entity shall maintain a reassembly window according to state variable RX_Next_Highest as follows:

-	a SN falls within the reassembly window if (RX_Next_Highest – UM_Window_Size) <= SN <RX_Next_Highest;

-	a SN falls outside of the reassembly window otherwise.

When receiving an UMD PDU from lower layer, the receiving UM RLC entity shall:

-	either deliver the UMD PDU to upper layer after removing the RLC header, discard the received UMD PDU, or place it in the reception buffer (see clause 5.2.2.2.2);

-	if the received UMD PDU was placed in the reception buffer:

-	update state variables, reassemble and deliver RLC SDUs to upper layer and start/stop t-Reassembly as needed (see clause 5.2.2.2.3).

When t-Reassembly expires, the receiving UM RLC entity shall:

-	update state variables, discard RLC SDU segments and start t-Reassembly as needed (see clause 5.2.2.2.4).

[TS 38.322, clause 5.2.2.2.3]

When an UMD PDU with SN = x is placed in the reception buffer, the receiving UM RLC entity shall:

-	if all byte segments with SN = x are received:

-	reassemble the RLC SDU from all byte segments with SN = x, remove RLC headers and deliver the reassembled RLC SDU to upper layer;

-	if x = RX_Next_Reassembly:

-	update RX_Next_Reassembly to the SN of the first SN > current RX_Next_Reassembly that has not been reassembled and delivered to upper layer.

-	else if x falls outside of the reassembly window:

-	update RX_Next_Highest to x + 1;

-	discard any UMD PDUs with SN that falls outside of the reassembly window;

-	if RX_Next_Reassembly falls outside of the reassembly window:

-	set RX_Next_Reassembly to the SN of the first SN >= (RX_Next_Highest – UM_Window_Size) that has not been reassembled and delivered to upper layer.

-	if t-Reassembly is running:

-	if RX_Timer_Trigger <= RX_Next_Reassembly; or

-	if RX_Timer_Trigger falls outside of the reassembly window and RX_Timer_Trigger is not equal to RX_Next_Highest; or

-	if RX_Next_Highest = RX_Next_Reassembly + 1 and there is no missing byte segment of the RLC SDU associated with SN = RX_Next_Reassembly before the last byte of all received segments of this RLC SDU:

-	stop and reset t-Reassembly.

-	if t-Reassembly is not running (includes the case when t-Reassembly is stopped due to actions above):

-	if RX_Next_Highest > RX_Next_Reassembly + 1; or

-	if RX_Next_Highest = RX_Next_Reassembly + 1 and there is at least one missing byte segment of the RLC SDU associated with SN = RX_Next_Reassembly before the last byte of all received segments of this RLC SDU:

-	start t-Reassembly;

-	set RX_Timer_Trigger to RX_Next_Highest.

[TS 38.322, clause 5.2.2.2.4]

When t-Reassembly expires, the receiving UM RLC entity shall:

-	update RX_Next_Reassembly to the SN of the first SN >= RX_Timer_Trigger that has not been reassembled;

-	discard all segments with SN < updated RX_Next_Reassembly;

-	if RX_Next_Highest > RX_Next_Reassembly + 1; or

-	if RX_Next_Highest = RX_Next_Reassembly + 1 and there is at least one missing byte segment of the RLC SDU associated with SN = RX_Next_Reassembly before the last byte of all received segments of this RLC SDU:

-	start t-Reassembly;

-	set RX_Timer_Trigger to RX_Next_Highest.

[TS 38.322, clause 7.1]

This clause describes the state variables used in AM and UM entities in order to specify the RLC protocol. The state variables defined in this clause are normative.

All state variables and all counters are non-negative integers.

…

All state variables related to UM data transfer can take values from 0 to 63 for 6 bit SN or from 0 to 4095 for 12 bit SN. All arithmetic operations contained in the present document on state variables related to UM data transfer are affected by the UM modulus (i.e. final value = [value from arithmetic operation] modulo 64 for 6 bit SN and 4096 for 12 bit SN).

When performing arithmetic comparisons of state variables or SN values, a modulus base shall be used.

…

RX_Next_Highest– UM_Window_Size shall be assumed as the modulus base at the receiving UM RLC entity. This modulus base is subtracted from all the values involved, and then an absolute comparison is performed (e.g. (RX_Next_Highest– UM_Window_Size) <= SN < RX_Next_Highest is evaluated as [(RX_Next_Highest– UM_Window_Size) – (RX_Next_Highest– UM_Window_Size)] modulo 2[sn-FieldLength] <= [SN – (RX_Next_Highest– UM_Window_Size)] modulo 2[sn-FieldLength] < [RX_Next_Highest– (RX_Next_Highest– UM_Window_Size)] modulo 2[sn-FieldLength]), where sn-FieldLength is 6 or 12 for 6 bit SN and 12 bit SN, respectively.

…

Each transmitting UM RLC entity shall maintain the following state variables:

a) TX_Next – UM send state variable

This state variable holds the value of the SN to be assigned for the next newly generated UMD PDU with segment. It is initially set to 0, and is updated after the UM RLC entity submits a UMD PDU including the last segment of an RLC SDU to lower layers.

Each receiving UM RLC entity shall maintain the following state variables:

a) RX_Next_Reassembly – UM receive state variable

This state variable holds the value of the earliest SN that is still considered for reassembly. It is initially set to 0. For groupcast and broadcast of NR sidelink communication or for SL-SRB4 of NR sidelink discovery, it is initially set to the SN of the first received UMD PDU containing an SN. For the receiving UM RLC entity configured for MCCH or MTCH, it is up to UE implementation to set the initial value of RX_Next_Reassembly to a value before RX_Next_Highest.

b) RX_Timer_Trigger – UM t-Reassembly state variable

This state variable holds the value of the SN following the SN which triggered t-Reassembly.

c) RX_Next_Highest– UM receive state variable

This state variable holds the value of the SN following the SN of the UMD PDU with the highest SN among received UMD PDUs. It serves as the higher edge of the reassembly window. It is initially set to 0. For groupcast and broadcast of NR sidelink communication or for SL-SRB4 of NR sidelink discovery, it is initially set to the SN of the first received UMD PDU containing an SN. For the receiving UM RLC entity configured for MCCH or MTCH, it is initially set to the SN of the first received UMD PDU containing an SN.

[TS 38.331, clause 6.3.2]

t-Reassembly, t-ReassemblyExt

Timer for reassembly in TS 38.322 [4], in milliseconds. Value ms0 means 0 ms, value ms5 means 5 ms and so on. If t-ReassemblyExt-r17 is configured, the UE shall ignore t-Reassembly (without suffix).

7.1.2.2.5a.3	Test description

7.1.2.2.5a.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.2.1.2 with the exception that the UM DRB is configured according to Table 7.1.2.2.5a.3.1-1.

Table 7.1.2.2.5a.3.1-1: RLC parameters

| t-Reassembly | ms200 |
| --- | --- |
| t-ReassemblyExt-r17 | ms1100 |
| Uplink UM RLC sn-FieldLength | IF (pc_um_WithShortSN) size6 ELSE size12 |
| Downlink UM RLC sn-FieldLength | IF (pc_um_WithShortSN) size6 ELSE size12 |

System Simulator:

-	NR Cell 1 as specified in TS 38.508-1 [4] Table 4.4.2.1.

-	System information combination NR-29 29 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used.

UE:

-	The pre-configured UE location is defined in TS 38.508-1 [4] Clause 4.5C.

7.1.2.2.5a.3.2	Test procedure sequence

Table 7.1.2.2.5a.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits UMD PDU#1 containing first segment of RLC SDU#1, SN=0. | <-- | UMD PDU#1 | - | - |
| 2 | Wait for t-Reassembly to ensure that t-Reassembly for the UMD PDU#1 expires. | - | - | - | - |
| 3 | The SS transmits UMD PDU#2 containing last segment of RLC SDU#1, SN=0. | <-- | UMD PDU#2 | - | - |
| 4 | Check: For 2 sec after step 3, does the UE transmit RLC SDU#1? (Note 2) | --> | (RLC SDU#1) | 1 | P |
| 5 | The SS transmits UMD PDU#3 containing first segment of RLC SDU#2, SN=2. | <-- | UMD PDU#3 | - | - |
| 6 | Wait for t-ReassemblyExt-r17 to ensure that t-ReassemblyExt-r17 for the UMD PDU#3 expires. | - | - | - | - |
| 7 | The SS transmits UMD PDU#4 containing last segment of RLC SDU#2, SN=2. | <-- | UMD PDU#4 | - | - |
| 8 | Check: For 2 sec after step 7, does the UE transmit RLC SDU#2? (Note 3) | --> | (RLC SDU#2) | 1 | F |
| Note 1: The RLC SDU size shall be 12 octets which are segmented into 7 and 5 octets. Note 2: The wait time of 1 sec is chosen to be greater than the propagation delay.The wait time of 2 sec is chosen to be greater than the propagation delays and transmissions of Scheduling Request and RLC SDU#1 if it is sent correctly. Note 3: The wait time of 2 sec is chosen to be greater than the propagation delays and transmissions of Scheduling Request and RLC SDU#2. |  |  |  |  |  |

7.1.2.2.5a.3.3	Specific message contents

Table 7.1.2.2.5a.3.3-1: RLC-Config (Preamble and all steps)

| Derivation Path: 38.508-1 [4], Table 4.6.3-149 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-Config ::= SEQUENCE { |  |  |  |
| um-Bi-Directional SEQUENCE { |  |  |  |
| dl-UM-RLC SEQUENCE { |  |  |  |
| t-Reassembly | ms200 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.2.2.5a.3.3-2: RLC-BearerConfig (Preamble and all steps)

| Derivation Path: 38.508-1 [4], Table 4.6.3-148 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-Config-v1700 ::= SEQUENCE { |  |  |  |
| DL-UM-RLC-v1700 SEQUENCE { |  |  |  |
| t-ReassemblyExt-r17 | ms1100 |  |  |
| } |  |  |  |
| } |  |  |  |

##### 7.1.2.2.6	UM RLC / RLC re-establishment procedure

7.1.2.2.6.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and using UM RLC }

ensure that {

when { RLC re-establishment is performed upon request by RRC }

then { The UE discards all UMD PDUs where no RLC SDUs can be reassembled }

}

(2)

with { UE in RRC_CONNECTED state and using UM RLC }

ensure that {

when { RLC re-establishment is performed upon request by RRC }

then { The UE resets variables TX_Next, RX_Next_Reassembly, and RX_Next_Highest to their initial value of 0 }

}

7.1.2.2.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.322, clauses 5.1.2 and 7.1, TS 38.331 clause 5.3.5.5.4. Unless otherwise stated these are Rel-15 requirements.

[TS 38.322, clause 5.1.2]

When upper layers request an RLC entity re-establishment, the UE shall:

-	discard all RLC SDUs, RLC SDU segments, and RLC PDUs, if any;

-	stop and reset all timers;

-	reset all state variables to their initial values.

[TS 38.322, clause 7.1]

d) RX_Next_Highest – Highest received state variable

This state variable holds the value of the SN following the SN of the RLC SDU with the highest SN among received RLC SDUs. It is initially set to 0.

Each transmitting UM RLC entity shall maintain the following state variables:

a) TX_Next

This state variable holds the value of the SN to be assigned for the next newly generated UMD PDU with segment. It is initially set to 0, and is updated after the UM RLC entity submits a UMD PDU including the last segment of an RLC SDU to lower layers.

Each receiving UM RLC entity shall maintain the following state variables and constant:

b) RX_Next_Reassembly – UM receive state variable

This state variable holds the value of the earliest SN that is still considered for reassembly. It is initially set to 0.

c) RX_Timer_Trigger – UM t-Reassembly state variable

This state variable holds the value of the SN following the SN which triggered t-Reassembly.

d) RX_Next_Highest– UM receive state variable

This state variable holds the value of the SN following the SN of the UMD PDU with the highest SN among received UMD PDUs. It serves as the higher edge of the reassembly window. It is initially set to 0.

[TS 38.331, clause 5.3.5.5.4]

For each RLC-Bearer-Config received in the rlc-BearerToAddModList IE the UE shall:

1>	if the UE’s current configuration contains a RLC bearer with the received logicalChannelIdentity:

2>	if reestablishRLC is received:

3>	re-establish the RLC entity as specified in TS 38.322 [4];

2>	reconfigure the RLC entity or entities in accordance with the received rlc-Config;

2>	reconfigure the logical channel in accordance with the received mac-LogicalChannelConfig;

NOTE:	The network does not re-associate an already configured logical channel with another radio bearer. Hence servedRadioBearer is not present in this case.

7.1.2.2.6.3	Test description

7.1.2.2.6.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.2.1.2 with the exception that the UM DRB is configured according to Table 7.1.2.2.6.3.1-1.

Table 7.1.2.2.6.3.1-1: RLC parameters

| t-Reassembly | ms200 |
| --- | --- |
| Uplink UM RLC sn-FieldLength | IF (pc_um_WithShortSN ) size6 ELSE size12 |
| Downlink UM RLC sn-FieldLength | IF (pc_um_WithShortSN ) size6 ELSE size12 |

7.1.2.2.6.3.2	Test procedure sequence

Table 7.1.2.2.6.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits UMD PDU#1. Header of UMD PDU#1 does not contain an SN. This PDU carries RLC SDU#1. | <-- | UMD PDU#1 | - | - |
| 2 | The UE transmits RLC SDU#1. | --> | (RLC SDU#1) | - | - |
| 3 | The SS transmits UMD PDU#2. Header of UMD PDU#2 contains SN=0. This PDU carries the first segment of SDU#2. | <-- | UMD PDU#2 | - | - |
| 4 | The SS transmits NR RRCReconfiguration message to trigger RLC re-establishment on DRB using Reconfig with sync procedure. (Note 3)(Note 5) | <-- | RRCReconfiguration | - | - |
| 4A | The UE transmits a NR RRCReconfigurationcomplete message. (Note 4) | --> | RRCReconfigurationComplete | - | - |
| 5 | 100 ms after step 4A the SS transmits UMD PDU#3. Header of UMD PDU#3 contains SN=0. This PDU carries the last segment of RLC SDU#2. The UE starts t-Reassembly. | <-- | UMD PDU#3 | - | - |
| 6 | Check: For 250 ms after step 5 does the UE transmit RLC SDU#2? | --> | (RLC SDU#2) | 1 | F |
| 6A | The SS stops allocating any UL grant. |  |  |  |  |
| 7 | 300 ms (1.5 * t- Reassembly) after step 5 the SS transmits UMD PDU#4. This PDU carries the first segment of RLC SDU#3.SN=1. | <-- | UMD PDU#4 | - | - |
| 8 | 20 ms after step 7 the SS transmits UMD PDU#5. This PDU carries the second and last segment of RLC SDU#3.SN=1. | <-- | UMD PDU#5 | - | - |
| 8A | 20 ms after step 8 the SS allocates 2 UL grants at an interval of 20 ms so as to loop back RLC SDU#3 in 2 RLC/MAC PDUs. Note 1 & 2 | - | - | - | - |
| 9 | Check: Does the UE transmit first segment of RLC SDU#3? Header of UMD PDU contains SN=0. | --> | (RLC SDU#3 first segment) | 2 | P |
| 10 | Check: Does the UE transmit second and last segment of RLC SDU#3? Header of UMD PDU contains SN=0. | --> | (RLC SDU#3 last segment) | 2 | P |
| 10A | The SS starts the UL default grant transmissions |  |  |  |  |
| 11 | The SS transmits NR RRCReconfiguration message to trigger RLC re-establishment on DRB using Reconfig with sync procedure. (Note 3) | <-- | RRCReconfiguration | - | - |
| 11A | The UE transmits a NR RRCReconfigurationcomplete message. (Note 4) | --> | RRCReconfigurationComplete | - | - |
| 11B | The SS stops allocating any UL grant. |  |  |  |  |
| 12 | After 100 ms the SS transmits UMD PDU#6. Header of UMD PDU#6 contains SN=0. This PDU carries the first segment of SDU#4. | <-- | UMD PDU#6 | - | - |
| 13 | 20 ms after step 12 the SS transmits UMD PDU#7. Header of UMD PDU#6 contains SN=0. This PDU carries the second segment of SDU#4. | <-- | UMD PDU#7 | - | - |
| 13A | 20 ms after step 13 the SS allocates 2 UL grants at an interval of 20 ms so as to loop back RLC SDU#4 in 2 RLC/MAC PDUs. Note 1 & 2 | - | - | - | - |
| 14 | Check: Does the UE transmit first segment of RLC SDU#4? Header of UMD PDU contains SN=0. | --> | (RLC SDU#4 first segment) | 2 | P |
| 15 | Check: Does the UE transmit second and last segment of RLC SDU#4? Header of UMD PDU contains SN=0. | --> | (RLC SDU#4 last segment) | 2 | P |
| Note 1: For SN size = size6 the RLC SDU size shall be 12 octets which are segmented into 6 and 6 octets. With 2 octets of MAC BSR and 2 octets of MAC header and 1 octet of RLC header (without SO) the first segment consists of 88 bits and a TBS of this size shall be allocated. With 2 octets of MAC header and 3 octets of RLC header (with SO) the second segment consists of 88 bits and a TBS of this size shall be allocated. (LRBs & IMCS as per 38.523-3[3] annex B) Note 2: For SN size = size12 the RLC SDU size shall be 10 octets which are segmented into 5 and 5 octets. With 2 octets of MAC BSR and 2 octets of MAC header and 2 octets of RLC header (without SO) the first segment consists of 88 bits and a TBS of this size shall be allocated. With 2 octets of MAC header and 4 octets of RLC header (with SO) the second segment consists of 88 bits and a TBS of this size shall be allocated. (LRBs & IMCS as per 38.523-3[3] annex B) Note 3: For EN-DC, the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration as defined in Table 7.1.2.2.6.3.3-2 Note 4: For EN-DC, the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. |  |  |  |  |  |

7.1.2.2.6.3.3	Specific message contents

Table 7.1.2.2.6.3.3-1: RRCReconfiguration for NR (steps 4, 11, Table 7.1.2.2.6.3.2-1)

| Derivation Path: 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig | According to Table 7.1.2.2.6.3.3-1A |  |
| } |  |  |  |
| nonCriticalExtension::= SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig | According to Table 7.1.2.2.6.3.3-1B |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.2.2.6.3.3-1A: RadioBearerConfig

| Derivation Path: 38.508-1 [4], Table 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | n entries | n is equal to the total number of DRBs established during preamble |  |
| DRB-ToAddMod[k=1..n] SEQUENCE { |  | entry (1..n) |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config | SDAP-Config | According to TS 38.508-1, Table 4.6.3-161 |  |
| } |  |  |  |
| drb-Identity | k | k=1..n |  |
| reestablishPDCP | Not present |  |  |
| recoverPDCP | Not present |  |  |
| pdcp-Config | PDCP-Config | According to TS 38.508-1, Table 4.6.3-99 |  |
| } |  |  |  |
| / } |  |  |  |
| } |  |  |  |

Table 7.1.2.2.6.3.3-1B: CellGroupConfig

| Derivation Path: 38.508-1 [4], Table 4.6.1-13 with condition PCell_change |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 2+n entries | n is equal to the total number of DRBs established during preamble |  |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions SRB1 and Re-establish_RLC | entry 1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions SRB2 and Re-establish_RLC | entry 2 |  |
| RLC-BearerConfig[3] | RLC-BearerConfig with conditions UM, DRB1 and Re-establish_RLC | entry 3 |  |
| RLC-BearerConfig[k+2, k=2..n] | RLC-BearerConfig with conditions AM, DRBk and Re-establish_RLC | entry [k+2, k=2..n] | n>1 |
| } |  |  |  |

Table 7.1.2.2.6.3.3-2: RRCConnectionReconfiguration for EN-DC (steps 4, 11 Table 7.1.2.2.6.3.2-1)

| Derivation Path: 36.508 Table 4.6.1-8 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  |  | Value/remark |  | Comment |  | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |  |  |  |
| rrcConnectionReconfiguration-r8 ::= SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |  |  |  |
| nr-Config-r15 CHOICE { |  |  |  |  |  |  |
| nr-SecondaryCellGroupConfig-r15 | OCTET STRING including the RRCReconfiguration message and the IE secondaryCellGroup according to TS 38.508-1 [67], table 4.6.1-13 with condition EN-DC_HO |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| nr-RadioBearerConfig1-r15 | OCTET STRING including RadioBearerConfig according to TS 38.508-1 [67], table 4.6.3-132 with conditions EN-DC_DRB |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

#### 7.1.2.3	RLC Acknowledged Mode

##### 7.1.2.3.1	AM RLC / 12-bit SN / Segmentation and reassembly / Segmentation Info (SI) field

7.1.2.3.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives a 12 bit SN configured AMD PDU containing a SI field set to 00 }

then { UE correctly decodes the received AMD PDU }

}

(2)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives a 12 bit SN configured AMD PDU containing a SI field set to 01 }

then { UE correctly decodes the received AMD PDU }

}

(3)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives a 12 bit SN configured AMD PDU containing a SI field set to 11 and SO field }

then { UE correctly decodes the received AMD PDU }

}

(4)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives a 12 bit SN configured AMD PDU containing a SI field set to 10 and SO field }

then { UE correctly decodes the received AMD PDU }

}

(5)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE has UL RLC SDU to send and the UL Grant is sufficient to send complete PDU }

then { UE transmits AMD PDU containing a complete AMD SDU and SI field set to 00 }

}

(6)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE has UL RLC SDU to send and the UL Grant is sufficient to send first segment only }

then { UE transmits AMD PDU containing first segment of AMD SDU and SI field set to 01 }

}

(7)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE has UL RLC SDU to send and the UL Grant is sufficient to send middle segment only }

then { UE transmits AMD PDU containing middle segment of AMD SDU and SI field set to 11, including SO field }

}

(8)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE has UL RLC SDU to send and the UL Grant is sufficient to send last segment only }

then { UE transmits AMD PDU containing last segment of AMD SDU and SI field set to 10, including SO field }

}

7.1.2.3.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.322, clauses 6.2.2.4 and 6.2.3.4. Unless otherwise stated these are Rel-15 requirements.

[TS 38.322, clause 6.2.2.4]

AMD PDU consists of a Data field and an AMD PDU header. The AMD PDU header is byte aligned.

An AM RLC entity is configured by RRC to use either a 12 bit SN or a 18 bit SN. The length of the AMD PDU header is two and three bytes respectively.

An AMD PDU header contains a D/C, a P, a SI, and a SN. An AMD PDU header contains the SO field only when the Data field consists of an RLC SDU segment which is not the first segment, in which case a 16 bit SO is present.

Figure 6.2.2.4-1: AMD PDU with 12 bit SN (No SO)

Figure 6.2.2.4-2: AMD PDU with 18 bit SN (No SO)

Figure 6.2.2.4-3: AMD PDU with 12 bit SN with SO

Figure 6.2.2.4-4: AMD PDU with 18 bit SN with SO

[TS 38.322, clause 6.2.3.4]

Length: 2 bits.

The SI field indicates whether an RLC PDU contains a complete RLC SDU or the first, middle, last segment of an RLC SDU.

Table 6.2.3.4-1: SI field interpretation

| Value | Description |
| --- | --- |
| 00 | Data field contains all bytes of an RLC SDU |
| 01 | Data field contains the first segment of an RLC SDU |
| 10 | Data field contains the last segment of an RLC SDU |
| 11 | Data field contains neither the first nor last segment of an RLC SDU |

7.1.2.3.1.3	Test description

7.1.2.3.1.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.2.1.1 with the exception that the AM DRB is configured according to Table 7.1.2.3.1.3.1-1.

Table 7.1.2.3.1.3.1-1: RLC parameters

| Uplink SN-FieldLength-AM | size12 |
| --- | --- |
| Downlink SN-FieldLength-AM | size12 |

7.1.2.3.1.3.2	Test procedure sequence

Table 7.1.2.3.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |  |
| 0 | The SS stops allocating any UL grant. | - | - | - | - |
| 1 | The SS transmits AMD PDU#1 containing a complete RLC SDU#1 (SI field = 00). | <-- | AMD PDU#1 | - | - |  |
| 1A | 60 ms after Step1, SS allocates an UL grant sufficient to loop back RLC SDU#1 in one RLC/MAC PDU | <-- | UL Grant | - | - |
| 2 | Check: Does the UE transmit AMD PDU#1 containing a complete RLC SDU#1 (SI field = 00)? | --> | (RLC SDU#1) | 1,5 | P |  |
| 3 | The SS transmits a STATUS PDU. | <-- | STATUS PDU (ACK SN=1) | - | - |  |
| 4 | The SS transmits AMD PDU#2 containing the first segment of RLC SDU#2 (SI field = 01). Note 3 | <-- | AMD PDU#2 | - | - |  |
| 5 | The SS transmits AMD PDU#3 containing the second segment of RLC SDU#2 (SI field = 11) and including SO field. Note 3 | <-- | AMD PDU#3 | - | - |  |
| 6 | The SS transmits AMD PDU#4 containing the last segment of RLC SDU#2 (SI field = 10) and including SO field. Note 3 | <-- | AMD PDU#4 | - | - |  |
| 6A | SS allocates 3 UL grants at an interval of 20 ms so as to loop back RLC SDU#2 in 3 RLC/MAC PDUs. (Note 1 and Note 2) | <-- | UL Grants | - | - |  |
| 7 | Check: Does the UE transmit AMD PDU#2 containing the first segment of RLC SDU#2 (SI field = 01)? | --> | (RLC SDU#2) | 2,3,4,6 | P |  |
| 8 | Check: Does the UE transmit AMD PDU#3 containing the middle segment of RLC SDU#2 (SI field = 11) and including SO field? | --> | (RLC SDU#2) | 2,3,4,7 | P |  |
| 9 | Check: Does the UE transmit AMD PDU#4 containing the last segment of RLC SDU#2 (SI field = 10) and including SO field? | --> | (RLC SDU#2) | 2,3,4,8 | P |  |
| 10 | The SS transmits a STATUS PDU. | <-- | STATUS PDU (ACK SN=2) | - | - |  |
| Note 1: The UL grants for step 7,8,9 are sufficiently small (240 bits, LRBs & IMCS as per 38.523-3[3] annex B) that UE transmits RLC SDU#2 in 3 UL RLC PDUs by segmenting. Note 2: The RLC PDU containing a segment shall be of size 208 bits resp. 224 bits and a MAC sub PDU header of 16 bits and a 16-bit MAC BSR CE included in step 8 resulting in a MAC PDU of size 240 bits. Note 3: The data part in step 4 first segment not including SO is 192 bits (24 bytes). Step 5, second segment SO=24 and data is 192 bits (24 bytes). Step 6, third segment SO=24+24=48 and data is 192 bits (24 bytes). |  |  |  |  |  |  |

7.1.2.3.1.3.3	Specific message contents

None

##### 7.1.2.3.2	AM RLC / 18-bit SN / Segmentation and reassembly / Segmentation Info (SI) field

7.1.2.3.2.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives a 18 bit SN configured AMD PDU containing a SI field set to 00 }

then { UE correctly decodes the received AMD PDU or AMD PDU segment }

}

(2)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives a 18 bit SN configured AMD PDU containing a SI field set to 01 }

then { UE correctly decodes the received AMD PDU or AMD PDU segment }

}

(3)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives a 18 bit SN configured AMD PDU containing a SI field set to 11 and SO field }

then { UE correctly decodes the received AMD PDU or AMD PDU segment }

}

(4)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives a 18 bit SN configured AMD PDU containing a SI field set to 10 and SO field }

then { UE correctly decodes the received AMD PDU or AMD PDU segment }

}

(5)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE has UL RLC SDU to send and the UL Grant is sufficient to send complete PDU }

then { UE transmits AMD PDU containing a complete AMD SDU and SI field set to 00 }

}

(6)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE has UL RLC SDU to send and the UL Grant is sufficient to send first segment only }

then { UE transmits AMD PDU containing first segment of AMD SDU and SI field set to 01  }

}

(7)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE has UL RLC SDU to send and the UL Grant is sufficient to send middle segment only }

then { UE transmits AMD PDU containing middle segment of AMD SDU and SI field set to 11, including SO field }

}

(8)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE has UL RLC SDU to send and the UL Grant is sufficient to send last segment only }

then { UE transmits AMD PDU containing last segment of AMD SDU and SI field set to 10, including SO field }

}

7.1.2.3.2.2	Conformance requirements

Same conformance requirements as in clause 7.1.2.3.1.2

7.1.2.3.2.3	Test description

7.1.2.3.2.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.2.1.1 with the exception that the AM DRB is configured according to Table 7.1.2.3.2.3.1-1.

Table 7.1.2.3.2.3.1-1: RLC parameters

| Uplink SN-FieldLength-AM | size18 |
| --- | --- |
| Downlink SN-FieldLength-AM | size18 |

7.1.2.3.2.3.2	Test procedure sequence

Same test procedure as in clause 7.1.2.3.1.3.2 except that SN is 18 bit and the data part in step 4 first segment not including SO is 184 bits (23 Bytes). Step 5, second segment SO=23 and data is 184 bits (23 bytes). Step 6, third segment SO=23+23=46 and data is 184 bits (23 bytes).

7.1.2.3.2.3.3	Specific message contents

None

##### 7.1.2.3.3	AM RLC / 12-bit SN / Correct use of sequence numbering

7.1.2.3.3.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with AM RLC 12 bit SN }
ensure that {

when { UE transmits the PDU corresponding to first SDU }
    then { UE includes the SN field equal to 0 in PDU }
            }

(2)

with { UE in RRC_CONNECTED state with AM RLC 12 bit SN }
ensure that {

when{ UE transmits subsequent SDUs }
    then { UE includes the SN field incremented by 1 per SDU of each PDU transmitted }
            }

(3)

with { UE in RRC_CONNECTED state with AM RLC 12 bit SN }
ensure that {

with { UE transmits more than 4096 SDUs}
    then { UE wraps the SN after transmitting the 4096 SDUs}

}

(4)

with { UE in RRC_CONNECTED state with AM RLC 12 bit SN
ensure that {

with { more than 4096 SDUs are sent to UE }
    then { UE accepts PDUs with SNs that wrap around every 4096 SDUs }

}

7.1.2.3.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.322, clauses 5.2.3.1.1, 5.2.3.2.1, 5.2.3.2.2, 6.2.2.4 and 7.1. Unless otherwise stated these are Rel-15 requirements.

[TS 38.322, clause 5.2.3.2.1]

The receiving side of an AM RLC entity shall maintain a receiving window according to the state variable RX_Next as follows:

-	a SN falls within the receiving window if RX_Next <= SN < RX_Next + AM_Window_Size;

-	a SN falls outside of the receiving window otherwise.

When receiving an AMD PDU from lower layer, the receiving side of an AM RLC entity shall:

-	either discard the received AMD PDU or place it in the reception buffer (see sub clause 5.2.3.2.2);

-	if the received AMD PDU was placed in the reception buffer:

-	update state variables, reassemble and deliver RLC SDUs to upper layer and start/stop t-Reassembly as needed (see sub clause 5.2.3.2.3).

When t-Reassembly expires, the receiving side of an AM RLC entity shall:

-	update state variables and start t-Reassembly as needed (see sub clause 5.2.3.2.4).

[TS 38.322, clause 5.2.3.2.2]

When an AMD PDU is received from lower layer, where the AMD PDU contains byte segment numbers y to z of an RLC SDU with SN = x, the receiving side of an AM RLC entity shall:

-	if x falls outside of the receiving window; or

-	if byte segment numbers y to z of the RLC SDU with SN = x have been received before:

-	discard the received AMD PDU.

-	else:

-	place the received AMD PDU in the reception buffer;

-	if some byte segments of the RLC SDU contained in the AMD PDU have been received before:

-	discard the duplicate byte segments.

[TS 38.322, clause 6.2.2.4]

AMD PDU consists of a Data field and an AMD PDU header. The AMD PDU header is byte aligned.

An AM RLC entity is configured by RRC to use either a 12 bit SN or a 18 bit SN. The length of the AMD PDU header is two and three bytes respectively.

An AMD PDU header contains a D/C, a P, a SI, and a SN. An AMD PDU header contains the SO field only when the Data field consists of an RLC SDU segment which is not the first segment, in which case a 16 bit SO is present.

Figure 6.2.2.4-1: AMD PDU with 12 bit SN (No SO)

Figure 6.2.2.4-2: AMD PDU with 18 bit SN (No SO)

Figure 6.2.2.4-3: AMD PDU with 12 bit SN with SO

Figure 6.2.2.4-4: AMD PDU with 18 bit SN with SO

[TS 38.322, clause 7.1]

c) RETX_COUNT – Counter

This counter counts the number of retransmissions of an RLC SDU or RLC SDU segment (see subclause 5.3.2). There is one RETX_COUNT counter maintained per RLC SDU.

The receiving side of each AM RLC entity shall maintain the following state variables:

a) RX_Next – Receive state variable

This state variable holds the value of the SN following the last in-sequence completely received RLC SDU, and it serves as the lower edge of the receiving window. It is initially set to 0, and is updated whenever the AM RLC entity receives an RLC SDU with SN = RX_Next.

b) RX_Next_Status_Trigger – t-Reassembly state variable

This state variable holds the value of the SN following the SN of the RLC SDU which triggered t-Reassembly.

c) RX_Highest_Status – Maximum STATUS transmit state variable

This state variable holds the highest possible value of the SN which can be indicated by "ACK_SN" when a STATUS PDU needs to be constructed. It is initially set to 0.

d) RX_Next_Highest – Highest received state variable

This state variable holds the value of the SN following the SN of the RLC SDU with the highest SN among received RLC SDUs. It is initially set to 0.

Each transmitting UM RLC entity shall maintain the following state variables:

a) TX_Next

This state variable holds the value of the SN to be assigned for the next newly generated UMD PDU with segment. It is initially set to 0, and is updated after the UM RLC entity submits a UMD PDU including the last segment of an RLC SDU to lower layers.

Each receiving UM RLC entity shall maintain the following state variables and constant:

b) RX_Next_Reassembly – UM receive state variable

This state variable holds the value of the earliest SN that is still considered for reassembly. It is initially set to 0.

c) RX_Timer_Trigger – UM t-Reassembly state variable

This state variable holds the value of the SN following the SN which triggered t-Reassembly.

d) RX_Next_Highest– UM receive state variable

This state variable holds the value of the SN following the SN of the UMD PDU with the highest SN among received UMD PDUs. It serves as the higher edge of the reassembly window. It is initially set to 0.

7.1.2.3.3.3	Test description

7.1.2.3.3.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.2.1.1 with the exception that the AM DRB is configured according to Table 7.1.2.3.3.3.1-1.

Table 7.1.2.3.3.3.1-1: RLC parameters

| Uplink SN-FieldLength-AM | size12 |
| --- | --- |
| Downlink SN-FieldLength-AM | size12 |
| pollPDU | p2048 |
| pollByte | kB25 |

7.1.2.3.3.3.2	Test procedure sequence

Table 7.1.2.3.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 1 – 3a1 shall be repeated from j=0 to j= FLOOR(Maximum_RLC_SN/iteration_size). (Note 1, 4, 6) | - | - | - | - |
| 1 | SS transmits in one slot, several RLC PDUs in a RLC PDU List, the number of RLC PDUs sent is defined by the iteration_size. (Note 4, 5). Each RLC PDU contains one RLC SDU. | <-- | RLC PDUs | - | - |
| - | EXCEPTION: In Step 2, SS may receive a RLC PDU or several RLC PDUs, then step 2 may be repeated multiple times until all RLC PDUs with SN=j*iteration_size to SN=(((j+1)*iteration_size)-1) for each iteration are received. | - | - | - | - |
| 2 | Check: Does UE transmit RLC PDUs with SN=0 for the first iteration and all RLC PDUs for each iteration? (Note 2) (Note 7) | --> | RLC PDUs | 1,2 | P |
| - | EXCEPTION: Step 3a1 describes behaviour that depends on the contents of the AMD PDU transmitted at Step 2. | - | - | - | - |
| 3a1 | IF the UE has set the poll bit in the AMD PDU transmitted at Step 2 THEN the SS transmits a Status Report. | <-- | STATUS PDU | - | - |
| 4 | SS transmits a RLC PDU containing one RLC SDU. | <-- | RLC PDU | - | - |
| 5 | Check: Does UE transmit a RLC PDU with SN=0? | --> | RLC PDU | 3,4 | P |
| 6 | The SS transmits a STATUS PDU with ACK_SN = 1. | <-- | STATUS PDU | - | - |
| Note 1: Maximum_RLC_SN = 2[RLC-SN-SizeUL] -1. Note 2: The verdict shall be provided each time [(SN+1) mod 256 = 0] for 12 bit SN and [(SN+1) mod 4096 = 0] for 18 bit SN respectively. Note 3: Void Note 4: Iteration will be incremented by iteration_size of 1 for 12 bit SN and iteration_size of 25 for 18 bit SN. Small RLC SDU size will be used. Note 5: SS shall transmit a RLC PDU list with size equal to iteration_size and incrementing SN by 1 till SN = ((j + 1) * iteration_size)-1. Note 6: The RLC SDU size shall be 4 octets for (pc_supportOfRedCap_r17=false AND pc_supportOfERedCap_r18= false) (3 octets of PDCP header + 1 octet PDCP SDU) or 4 octets for (pc_supportOfRedCap_r17=true OR pc_supportOfERedCap_r18=true) (2 octets of PDCP header + 2 octets PDCP SDU). Note 7: All RLC PDUs may be received by the SS in the same slot or in multiple slots (max one MAC PDU in a slot). |  |  |  |  |  |

7.1.2.3.3.3.3	Specific message contents

None.

##### 7.1.2.3.4	AM RLC / 18-bit SN / Correct use of sequence numbering

7.1.2.3.4.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with AM RLC 18 bit SN }
ensure that {

when { UE transmits the PDU corresponding to first SDU }
    then { UE includes the SN field equal to 0 in PDU }
            }

(2)

with { UE in RRC_CONNECTED state with AM RLC 18 bit SN }
ensure that {

when{ UE transmits subsequent SDUs }
    then { UE includes the SN field incremented by 1 per SDU of each PDU transmitted }
            }

(3)

with { UE in RRC_CONNECTED state with AM RLC 18 bit SN }
ensure that {

with { UE transmits more than 262144 SDUs }
    then { UE wraps the SN after transmitting the 262144 SDUs }

}

(4)

with { UE in RRC_CONNECTED state with AM RLC 18 bit SN
ensure that {

with { more than 262144 SDUs are sent to UE }
    then { UE accepts PDUs with SNs that wrap around every 262144 SDUs }

}

7.1.2.3.4.2	Conformance requirements

Same as conformance requirements in clause 7.1.2.3.3.2

7.1.2.3.4.3	Test description

7.1.2.3.4.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.2.1.1 with the exception that the AM DRB is configured according to Table 7.1.2.3.4.3.1-1.

Table 7.1.2.3.4.3.1-1: RLC parameters

| Uplink SN-FieldLength-AM | size18 |
| --- | --- |
| Downlink SN-FieldLength-AM | size18 |
| pollPDU | p2048 |
| pollByte | kB25 |

7.1.2.3.4.3.2	Test procedure sequence

Same as test procedure in clause 7.1.2.3.3.3.2

7.1.2.3.4.3.3	Specific message contents

None.

##### 7.1.2.3.5	AM RLC / 12-bit SN / Control of transmit window / Control of receive window

7.1.2.3.5.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with AM RLC 12 bit SN and pending uplink data for transmission }

ensure that {

when { AMD PDUs in transmission buffer fall outside TX_Next_Ack <= SN < TX_Next_Ack + AM_Window_Size }

then { UE does not transmit these AMD PDUs }

}

(2)

with { UE in RRC_CONNECTED state with AM RLC 12 bit SN and pending uplink data for transmission }

ensure that {

when { receiving a STATUS PDU where ACK_SN acknowledges at least one AMD PDU not yet acknowledged }

then { UE transmits AMD PDUs within updated window range }

}

(3)

with { UE in RRC_CONNECTED state with AM RLC 12 bit SN }

ensure that {

when { the UE receives AMD PDUs with SN outside the upper boundary of the receive window }

then { the UE discards these AMD PDUs }

}

(4)

with { UE in RRC_CONNECTED state with AM RLC 12 bit SN }

ensure that {

when { the receive window has been moved }

then { UE continues accepting AMD PDUs within updated window range }

}

7.1.2.3.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.322, clauses 5.2.3.2.1, 5.2.3.2.2, 5.2.3.2.3 and 7.2. Unless otherwise stated these are Rel-15 requirements.

[TS 38.322, clause 5.2.3.2.2]

When an AMD PDU is received from lower layer, where the AMD PDU contains byte segment numbers y to z of an RLC SDU with SN = x, the receiving side of an AM RLC entity shall:

-	if x falls outside of the receiving window; or

-	if byte segment numbers y to z of the RLC SDU with SN = x have been received before:

-	discard the received AMD PDU.

-	else:

-	place the received AMD PDU in the reception buffer;

-	if some byte segments of the RLC SDU contained in the AMD PDU have been received before:

-	discard the duplicate byte segments.

[TS 38.322, clause 5.2.3.2.3]

When an AMD PDU with SN = x is placed in the reception buffer, the receiving side of an AM RLC entity shall:

-	if x >= RX_Next_Highest

-	update RX_Next_Highest to x+ 1.

-	if all bytes of the RLC SDU with SN = x are received:

-	reassemble the RLC SDU from AMD PDU(s) with SN = x, remove RLC headers when doing so and deliver the reassembled RLC SDU to upper layer;

-	if x = RX_Highest_Status,

-	update RX_Highest_Status to the SN of the first RLC SDU with SN > current RX_Highest_Status for which not all bytes have been received.

-	if x = RX_Next:

-	update RX_Next to the SN of the first RLC SDU with SN > current RX_Next for which not all bytes have been received.

-	if t-Reassembly is running:

-	if RX_Next_Status_Trigger = RX_Next; or

-	if RX_Next_Status_Trigger = RX_Next + 1 and there is no missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU; or

-	if RX_Next_Status_Trigger falls outside of the receiving window and RX_Next_Status_Trigger is not equal to RX_Next + AM_Window_Size:

-	stop and reset t-Reassembly.

-	if t-Reassembly is not running (includes the case t-Reassembly is stopped due to actions above):

-	if RX_Next_Highest> RX_Next +1; or

-	if RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU:

-	start t-Reassembly;

-	set RX_Next_Status_Trigger to RX_Next_Highest.

[TS 38.322, clause 7.2]

a) AM_Window_Size

This constant is used by both the transmitting side and the receiving side of each AM RLC entity. AM_Window_Size = 2048 when a 12 bit SN is used, AM_Window_Size = 131072 when an 18 bit SN is used.

7.1.2.3.5.3	Test description

7.1.2.3.5.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.2.1.1 with the exception that the AM DRB is configured according to Table 7.1.2.3.5.3.1-1.

Table 7.1.2.3.5.3.1-1: RLC parameters

| t-PollRetransmit | ms300 |
| --- | --- |
| pollPDU | infinity |
| pollByte | infinity |
| sn-FieldLength(UL-AM-RLC) | size12 |
| sn-FieldLength(DL-AM-RLC) | size12 |

7.1.2.3.5.3.2	Test procedure sequence

Table 7.1.2.3.5.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | The SS does not allocate any uplink grant. | - | - | - | - |
| - | EXCEPTION: The SS is configured for step 1 500 ms in advance. The transmissions are performed every second radio frame. Step 2 is started 100 ms after the first DL AMD PDU has been transmitted in step 1. | - | - | - | - |
| - | EXCEPTION: Step 1 a1 shall be repeated from j=0 to j= FLOOR((Maximum_RLC_SN/iteration size) -1, and the last repetition shall execute Step 1b1. (Note 1) (Note 3) (Note 4) (Note 6) | - | - | - | - |
| 1a1 | The SS transmits several RLC PDUs in a RLC PDU List, the number of RLC PDUs sent is defined by the iteration_size. Each RLC Data PDU contains one RLC SDU. (Note 8) (Note 9) (Note 5) | <-- | RLC Data PDU (SN = j*iteration_ size, SN=(((j+1)*iteration_size)-1) | - | - |
| 1b1 | The SS transmits several RLC PDUs in a RLC PDU List, the number of RLC PDUs sent is defined by the iteration_size+1. Each RLC Data PDU contains one RLC SDU. The SS transmits AMD PDU(SN=W+1) as last packet (Note 8) (Note 9) (Note 5) | <-- | RLC Data PDU (SN = j*iteration_ size, SN=(((j+1)*iteration_size)) | - | - |
| 1A | Void | - | - | - | - |
| 2 | In the following steps the SS transmits 1 UL grant in every second radio frame to enable the UE to return each received AMD PDU in one looped back AMD PDU. (Note 2) | <-- | (UL grants) | - | - |
| - | EXCEPTION: Step 2A shall be repeated from j=0 to j=FLOOR((Maximum_RLC_SN/iteration size). (Note 1) (Note 3) (Note 4) (Note 6) | - | - | - | - |
| - | EXCEPTION: In Step 2A, SS shall receive a RLC PDU and step 2A is repeated from SN=j*iteration_size to SN=(((j+1)*iteration_ size)-1). (Note 1) (Note 3) (Note 4) (Note 6) (Note 8) (Note 9) | - | - | - | - |
| 2A | Check: Does UE transmit a RLC Data PDU with the Poll bit not set and with SN=0 for the first RLC Data PDU and then incremented by 1 at each RLC Data PDU? (Note 7)(Note 10) | --> | RLC Data PDU (SN = j*iteration_ size, SN=(((j+1)*iteration_size)-1) | 1 | P |
| 3 | Check: Does the UE transmit the (W)st AMD PDU with the Poll bit set and with the contents of the SDU? | --> | AMD PDU(SN=W-1), Poll | 1 | P |
| 4 | The SS starts the UL default grant transmission. | - | - | - | - |
| 5 | Check: Does the UE transmit an AMD PDU within t-PollRetransmit/2? | --> | AMD PDU | 1 | F |
| 6 | The SS transmits a STATUS PDU to acknowledge the W uplink AMD PDUs with SN=0 to SN=W-1. ACK_SN = W. | <-- | STATUS PDU | - | - |
| 7 | Check: Does the UE transmit an AMD PDU with the Poll bit set and with the contents of the SDU? | --> | AMD PDU(SN=W), Poll | 2 | P |
| 8 | The SS transmits a STATUS PDU with ACK_SN = W+1. | <-- | STATUS PDU | - | - |
| 9 | The SS transmits the (W+2)nd AMD PDU containing a SDU to the UE with the Sequence Number field set to ((2W+1 mod AM_Modulus) = 1) and the Polling bit set. (Note 3) (Note 5) | <-- | AMD PDU | - | - |
| 10 | Check: Does the UE transmit a STATUS PDU acknowledging W+1 SDUs? (ACK_SN = W+1). (Note 1) | --> | STATUS PDU | 3 | P |
| 11 | The SS transmits the (W+2)nd AMD PDU to the UE with the Sequence Number field set to W+1 and the Polling bit set. (Note 5) | <-- | AMD PDU | - | - |
| - | EXCEPTION: Steps 12 and 13 can happen in any order | - | - | - | - |
| 12 | Check: Does the UE transmit a STATUS PDU acknowledging W+1 PDUs? (ACK_SN field = W+2). (Note 11) | --> | STATUS PDU | 4 | P |
| 13 | Check: Does the UE transmit an AMD PDU with the same data as received in the corresponding DL AMD PDU in step 11? (Note 11) | --> | AMD PDU | 4 | P |
| 14 | The SS transmits a STATUS PDU with ACK_SN = W+2. | <-- | STATUS PDU | - | - |
| Note 1: PDUs are numbered 1,2, …, W+2. Note 2: 20 ms gap between transmissions both in DL and UL respectively allows TTCN to tolerate one HARQ retransmission (FDD/TDD) per transport block. Note 3: AM_Modulus is 4096 resp 262144 for SN size is size12 or size18. Note 4: The RLC SDU size shall be 4 octets(3 octets of PDCP header + 1 octet PDCP SDU) for (pc_supportOfRedCap_r17=false AND pc_supportOfERedCap_r18= false) or 4 octets(2 octets of PDCP header + 2 octets PDCP SDU) for (pc_supportOfRedCap_r17=true OR pc_supportOfERedCap_r18=true). If SN size is size18 is used the RLC SDU size shall be 7 octets. With 2 octets of BSR or padding, 2 octets of MAC header and 3 octets of RLC header (without SO) the RLC PDU consists of 56 bits and a TBS of 112 bits shall be allocated. Note 5: PDCP SN=W+1. Note 6: Maximum_RLC_SN = W-1. Note 7: The verdict shall be provided each time (SN+1) mod 256 = 0 resp. (SN+1) mod 4096 = 0, if SN size is size12 or size18. Note 8: Iteration will be 211 in case of len12bits. Small RLC SDU size will be used and no repetition will be needed. Note 9: Iteration will be incremented by iteration_size of 211 for SN len18bits. Small RLC SDU size will be used and it shall be repeated FLOOR(Maximum_RLC_SN/iteration_size). Note 10: -2 for the last iteration, as the last reception will be handled by step 3. Note 11: STATUS PDU at step 12 and AMD PDU at step 13 may be received by SS in the same slot or in multiple slots. |  |  |  |  |  |

7.1.2.3.5.3.3	Specific message contents

None

##### 7.1.2.3.5a	AM RLC / 18-bit SN / Control of transmit window / Control of receive window

7.1.2.3.5a.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with AM RLC 18 bit SN and pending uplink data for transmission }

ensure that {

when { AMD PDUs in transmission buffer fall outside TX_Next_Ack <= SN < TX_Next_Ack + AM_Window_Size }

then { UE does not transmit these AMD PDUs }

}

(2)

with { UE in RRC_CONNECTED state with AM RLC 18 bit SN and pending uplink data for transmission }

ensure that {

when { receiving a STATUS PDU where ACK_SN acknowledges at least one AMD PDU not yet acknowledged }

then { UE transmits AMD PDUs within updated window range }

}

(3)

with { UE in RRC_CONNECTED state with AM RLC 18 bit SN }

ensure that {

when { the UE receives AMD PDUs with SN outside the upper boundary of the receive window }

then { the UE discards these AMD PDUs }

}

(4)

with { UE in RRC_CONNECTED state with AM RLC 18 bit SN }

ensure that {

when { the receive window has been moved }

then { UE continues accepting AMD PDUs within updated window range }

}

7.1.2.3.5a.2	Conformance requirements

Same as conformance requirements in clause 7.1.2.3.5.2.

7.1.2.3.5a.3	Test description

7.1.2.3.5a.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.2.1.1 with the exception that the AM DRB is configured according to Table 7.1.2.3.5a.3.1-1.

Table 7.1.2.3.5a.3.1-1: RLC parameters

| t-PollRetransmit | ms300 |
| --- | --- |
| pollPDU | infinity |
| pollByte | infinity |
| sn-FieldLength(UL-AM-RLC) | size18 |
| sn-FieldLength(DL-AM-RLC) | size18 |

7.1.2.3.5a.3.2	Test procedure sequence

Same as test procedure in clause 7.1.2.3.5.3.2

7.1.2.3.5a.3.3	Specific message contents

None.

##### 7.1.2.3.6	AM RLC / Polling for status

7.1.2.3.6.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and using AM RLC }

ensure that {

when { last data in the UL buffer is being transmitted }

then { UE transmits a Poll }

}

(2)

with { UE in RRC_CONNECTED state and using AM RLC }

ensure that {

when { the t-PollRetransmit timer expires }

then { UE transmits a Poll }

}

(3)

with { UE in RRC_CONNECTED state and using AM RLC }

ensure that {

when { PDU_WITHOUT_POLL >= pollPDU }

then { UE transmits a Poll }

}

(4)

with { UE in RRC_CONNECTED state and using AM RLC }

ensure that {

when { BYTE_WITHOUT_POLL >= pollByte }

then { UE transmits a Poll }

}

7.1.2.3.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.322, clauses 5.3.3.2, 7.3 and 7.4. Unless otherwise stated these are Rel-15 requirements.

[TS 38.322, clause 5.3.3.2]

Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission such that the AMD PDU contains either a not previously transmitted RLC SDU or an RLC SDU segment containing not previously transmitted byte segment, the transmitting side of an AM RLC entity shall:

-	increment PDU_WITHOUT_POLL by one;

-	increment BYTE_WITHOUT_POLL by every new byte of Data field element that it maps to the Data field of the AMD PDU;

-	if PDU_WITHOUT_POLL >= pollPDU; or

-	if BYTE_WITHOUT_POLL >= pollByte:

-	include a poll in the AMD PDU as described below.

Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission, the transmitting side of an AM RLC entity shall:

-	if both the transmission buffer and the retransmission buffer becomes empty (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) after the transmission of the AMD PDU; or

-	if no new RLC SDU can be transmitted after the transmission of the AMD PDU (e.g. due to window stalling);

-	include a poll in the AMD PDU as described below.

NOTE:	Empty RLC buffer (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) should not lead to unnecessary polling when data awaits in the upper layer. Details are left up to UE implementation.

To include a poll in an AMD PDU, the transmitting side of an AM RLC entity shall:

-	set the P field of the AMD PDU to "1";

-	set PDU_WITHOUT_POLL to 0;

-	set BYTE_WITHOUT_POLL to 0.

After submitting an AMD PDU including a poll to lower layer and after incrementing of TX_Next if necessary, the transmitting side of an AM RLC entity shall:

-	set POLL_SN to TX_Next – 1;

-	if t-PollRetransmit is not running:

-	start t-PollRetransmit.

-	else:

-	restart t-PollRetransmit.

[TS 38.322, clause 5.3.3.4]

Upon expiry of t-PollRetransmit, the transmitting side of an AM RLC entity shall:

-	if both the transmission buffer and the retransmission buffer are empty (excluding transmitted RLC SDU or RLC SDU segment awaiting acknowledgements); or

-	if no new RLC SDU or RLC SDU segment can be transmitted (e.g. due to window stalling):

-	consider the RLC SDU with SN = TX_Next – 1 for retransmission; or

-	consider any RLC SDU which has not been positively acknowledged for retransmission.

-	include a poll in an AMD PDU as described in section 5.3.3.2.

[TS 38.322, clause 7.3]

a) t-PollRetransmit

This timer is used by the transmitting side of an AM RLC entity in order to retransmit a poll (see sub clause 5.3.3).

[TS 38.322, clause 7.4]

b) pollPDU

This parameter is used by the transmitting side of each AM RLC entity to trigger a poll for every pollPDU PDUs (see subclause 5.3.3).

c) pollByte

This parameter is used by the transmitting side of each AM RLC entity to trigger a poll for every pollByte bytes (see subclause 5.3.3).

7.1.2.3.6.3	Test description

7.1.2.3.6.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.2.1.1 with the exception that the AM DRB is configured according to Table 7.1.2.3.6.3.1-1.

Table 7.1.2.3.6.3.1-1: RLC parameters

| t-PollRetransmit | ms400 |
| --- | --- |
| pollPDU | p256 |
| pollByte | kB25 |

7.1.2.3.6.3.2	Test procedure sequence

Table 7.1.2.3.6.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | During the whole test sequence, the SS should not allocate UL grants unless when explicitly stated so in the procedure. | - | - | - | - |
| 2 | The SS transmits 4 AMD PDUs such that 1 AMD PDU is sent every two radio frame, each containing an RLC SDU. (Note 2) | <-- | AMD PDU (SN=0) AMD PDU (SN=1) AMD PDU (SN=2) AMD PDU (SN=3) | - | - |
| - | EXCEPTION: In parallel to the events described in step 3, the step specified in Table 7.1.2.3.6.3.2-2 should take place. | - | - | - | - |
| 3 | The SS waits for 100 ms after the first DL AMD PDU has been transmitted in step 2, then starts assigning UL grants in every second radio frame of size 1032 bits. (Note 1) (Note 2) | - | - | - | - |
| 4 | Check 1: Does the UE transmit an AMD PDU with a SN in range 0 to 3 and P=1? Record time TB. Check 2: Is (TB – TA) = t-PollRetransmit? | --> | AMD PDU | 2 | P |
| 5 | The SS starts the UL default grant transmission on reception of SR. | - | - | - | - |
| 6 | The SS transmits an RLC Status Report ACKing reception of PDU’s 0-3. | <-- | STATUS PDU | - | - |
| 7 | Check: Does the UE retransmit an AMD PDU within 1 sec? | --> | AMD PDU | 2 | F |
| 8 | The SS transmits NR RRCReconfiguration message changing pollPDU to p4. (Note 3) | <-- | RRCReconfiguration | - | - |
| 8A | The UE transmits a NR RRCReconfigurationcomplete message. (Note 4) | --> | RRCReconfigurationComplete | - | - |
| 9 | The SS stops allocating any UL grant. | - | - | - | - |
| 10 | The SS transmits 8 AMD PDUs such that 1 AMD PDU is sent every second radio frame, each containing an RLC SDU. (Note 2) | <-- | AMD PDU (SN=4) AMD PDU (SN=5) ... AMD PDU (SN=11) | - | - |
| - | EXCEPTION: In parallel to the events described in step 11, the step specified in Table 7.1.2.3.6.3.2-3 should take place. | - | - | - | - |
| 11 | The SS waits for 100 ms after the first DL AMD PDU has been transmitted in step 10, then starts assigning UL grants (UL grant allocation type 2) in every second radio frame of size 1032 bits. (Note 1) (Note 2) | - | - | - | - |
| 12 | The SS transmits a Status Report with ACK_SN=12, NACK_SN=4, NACK_SN=5, NACK_SN=6 (constructed by NACK_SN Range), NACK_SN=8 and NACK_SN=9 (constructed by NACK_SN Range). | <-- | STATUS PDU | - | - |
| 12A | Void. | - | - | - | - |
| 13 | Check: Does the UE transmit AMD PDUs with the following SN and P values? AMD PDU, SN=4, P=0 AMD PDU, SN=5, P=0 AMD PDU, SN=6, P=0 AMD PDU, SN=8, P=0 AMD PDU, SN=9, P=1 | --> | AMD PDU (SN=4, P=0) AMD PDU (SN=5, P=0) AMD PDU (SN=6, P=0) AMD PDU (SN=8, P=0) AMD PDU (SN=9, P=1) | 2 | P |
| 14 | The SS starts the UL default grant transmission on reception of SR. | - | - | - | - |
| 15 | The SS transmits a Status Report with ACK_SN=12 and no NACK_SN. | <-- | STATUS PDU | - | - |
| 16 | The SS transmits NR RRCReconfiguration message changing pollPDU to p256. (Note 3) | <-- | RRCReconfiguration | - | - |
| 16A | The UE transmits a NR RRCReconfigurationcomplete message. (Note 4) | --> | RRCReconfigurationComplete | - | - |
| 17 | The SS does not allocate any UL grant. | - | - | - | - |
| 18 | After 500 ms the SS transmits 412 AMD PDUs such that 1 AMD PDU is sent every second radio frame, each containing an RLC SDU. (Note 2) | <-- | AMD PDU (SN=12) AMD PDU (SN=13) ... AMD PDU (SN=423) | - | - |
| - | EXCEPTION: In parallel to the events described in step 19, the steps specified in Table 7.1.2.3.6.3.2-4 should take place. | - | - | - | - |
| 19 | The SS waits for 100 ms after the first DL AMD PDU has been transmitted in step 10, then starts assigning UL grants (UL grant allocation type 2) in every second radio frame of size 1032 bits. (Note 1) (Note 2) | - | - | - | - |
| 20 | The SS starts the UL default grant transmission | - | - | - | - |
| Note 1: UL grant of 1032 bits (LRBs & IMCS as per 38.523-3[3] annex B) is chosen to allow the UE to loop back one SDU of size 976 bits and one short BSR (16 bits) into each MAC PDU sent in the uplink (1032 bits - 24 bit AMD PDU header - 16 bit MAC BSR CE- 16 bit MAC PDU subheader) for (pc_supportOfRedCap_r17=false AND pc_supportOfERedCap_r18= false), or one SDU of size 976 bits and one short BSR (16 bits) and/or padding into each MAC PDU sent in the uplink (1032 bits - 16 bit AMD PDU header - 16 bit MAC PDU subheader - 24 bit (MAC BSR CE and/or Padding)) for (pc_supportOfRedCap_r17=true OR pc_supportOfERedCap_r18=true). The UE will include an SDU and one short BSR in the looped back MAC PDU. Note 2: 20ms gap between transmissions both in DL and UL respectively allows TTCN to tolerate one HARQ retransmission (FDD/TDD) per transport block, if such happen (TS 38.523-3 [3]). Note 3: For EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration 36.508 [7] Table 4.6.1-8 using condition EN-DC_EmbedNR_RRCRecon. Note 4: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. |  |  |  |  |  |

Table 7.1.2.3.6.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE transmit 4 AMD PDUs, with only the last one having the poll bit set? Record time TA when the PDU with the poll bit set is received at the SS. | --> | AMD PDUs | 1 | P |

Table 7.1.2.3.6.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE transmit 8 AMD PDUs, with the poll bit set only in the 4th and the 8th PDUs? | --> | AMD PDUs | 3 | P |

Table 7.1.2.3.6.3.2-4: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE transmit 205 AMD PDUs, with the poll bit set only in the last (205th) one? (Note 1) | --> | AMD PDUs | 4 | P |
| 2 | The SS transmits an RLC Status Report. | <-- | STATUS PDU | - | - |
| 3 | Check: Does the UE transmit 205 AMD PDUs, with the poll bit set only in the last (410th) one? (Note 1) | --> | AMD PDUs | 4 | P |
| 4 | The SS transmits an RLC Status Report. | <-- | STATUS PDU | - | - |
| 5 | Check: Does the UE transmit 2 AMD PDUs, with the poll bit set only in the last (412th ) one? | --> | AMD PDUs | 1 | P |
| 6 | The SS transmits an RLC Status Report. | <-- | STATUS PDU | - | - |
| Note 1: (976 bits x 205PDUs) / 8 = 25010 > 25 KB, with 1 kB = 1000 bytes (TS 38.331 [12], clause 3.2) |  |  |  |  |  |

7.1.2.3.6.3.3	Specific message contents

Table 7.1.2.3.6.3.3-1: RRCReconfiguration (steps 8 and 16, Table 7.1.2.3.6.3.2-1)

| Derivation Path: 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| secondaryCellGroup | CellGroupConfig |  | EN-DC |
| nonCriticalExtension SEQUENCE { |  |  | NR |
| masterCellGroup | CellGroupConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.2.3.6.3.3-2: CellGroupConfig (Table 7.1.2.3.6.3.3-1)

| Derivation Path: 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 1 entry |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig | entry 1 |  |
| } |  |  |  |
| mac-CellGroupConfig | Not present |  |  |
| physicalCellGroupConfig | Not present |  |  |
| spCellConfig | Not present |  |  |
| } |  |  |  |

Table 7.1.2.3.6.3.3-3: RLC-BearerConfig (Table 7.1.2.3.6.3.3-2)

| Derivation Path: 38.508-1 [4], Table 4.6.3-148 with condition AM |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-BearerConfig ::= SEQUENCE { |  |  |  |
| logicalChannelIdentity | Set to LCID of the DRB under test |  |  |
| rlc-Config | RLC-Config |  |  |
| } |  |  |  |

Table 7.1.2.3.6.3.3-4: RLC-Config (Table 7.1.2.3.6.3.3-3)

| Derivation Path: 38.508-1 [4], Table 4.6.3-149 with condition AM |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-Config ::= CHOICE { |  |  |  |
| am SEQUENCE { |  |  |  |
| ul-AM-RLC SEQUENCE { |  |  |  |
| sn-FieldLength | Not present |  |  |
| t-PollRetransmit | ms400 |  |  |
| pollPDU | p4 |  | step 8 |
|  | p256 |  | step 16 |
| pollByte | kB25 |  |  |
| } |  |  |  |
| dl-AM-RLC SEQUENCE { |  |  |  |
| sn-FieldLength | Not present |  |  |
| } |  |  |  |

##### 7.1.2.3.7	AM RLC / Receiver status triggers

7.1.2.3.7.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and using AM RLC }

ensure that {

when { Reception failure of an RLC data PDU is detected and t-Reassembly expires }

then { UE initiates Status Reporting }

}

(2)

with { UE in RRC_CONNECTED state and using AM RLC }

ensure that {

when { Status Reporting is triggered and t-StatusProhibit is running }

then { UE wait until t-StatusProhibit has expired to send Status Report}

}

(3)

with { UE in RRC_CONNECTED state and using AM RLC }

ensure that {

when { Polling from peer AM RLC entity is detected and the sequence number ‘x’ of the PDU that carries the Poll satisfies x < RX_Highest_Status or x >= RX_Next + AM_Window_Size }

then { UE initiates Status Reporting }

}

(4)

with { UE in RRC_CONNECTED state and using AM RLC }

ensure that {

when { Polling from peer AM RLC entity is detected and the sequence number ‘x’ of the PDU that carries the Poll does not satisfies x < RX_Highest_Status or x >= RX_Next + AM_Window_Size }

then { UE waits until ‘x < RX_Highest_Status or x >= RX_Next + AM_Window_Size’ before initiating Status Reporting}

}

(5)

with { UE in RRC_CONNECTED state and using AM RLC }

ensure that {

when { the UE needs to send a Status Report and the UL grant is not large enough to accommodate the whole report }

then { UE includes as many NACK_SNs in the Status Report as allowed by the UL grant }

}

(6)

with { UE in RRC_CONNECTED state and using AM RLC }

ensure that {

when { the UE needs to send a Status Report and continuous sequence of RLC SDUs that have not been received yet }

then { UE includes NACK_SN with NACK range }

}

7.1.2.3.7.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.322, clause 5.3.4. Unless otherwise stated these are Rel-15 requirements.

[TS 38.322, clause 5.3.4]

An AM RLC entity sends STATUS PDUs to its peer AM RLC entity in order to provide positive and/or negative acknowledgements of RLC SDUs (or portions of them).

Triggers to initiate STATUS reporting include:

-	Polling from its peer AM RLC entity:

-	When an AMD PDU with SN = x and the P field set to "1" is received from lower layer, the receiving side of an AM RLC entity shall:

-	if the AMD PDU is to be discarded as specified in subclause 5.2.3.2.2; or

-	if x < RX_Highest_Status or x >= RX_Next + AM_Window_Size:

-	trigger a STATUS report.

-	else:

-	delay triggering the STATUS report until x < RX_Highest_Status or x >= RX_Next + AM_Window_Size.

NOTE 1:	This ensures that the RLC Status report is transmitted after HARQ reordering.

-	Detection of reception failure of an AMD PDU

-	The receiving side of an AM RLC entity shall trigger a STATUS report when t-Reassembly expires.

NOTE 2:	The expiry of t-Reassembly triggers both RX_Highest_Status to be updated and a STATUS report to be triggered, but the STATUS report shall be triggered after RX_Highest_Status is updated.

When STATUS reporting has been triggered, the receiving side of an AM RLC entity shall:

-	if t-StatusProhibit is not running:

-	at the first transmission opportunity indicated by lower layer, construct a STATUS PDU and submit it to lower layer.

-	else:

-	at the first transmission opportunity indicated by lower layer after t-StatusProhibit expires, construct a single STATUS PDU even if status reporting was triggered several times while t-StatusProhibit was running and submit it to lower layer.

When a STATUS PDU has been submitted to lower layer, the receiving side of an AM RLC entity shall:

-	start t-StatusProhibit.

When constructing a STATUS PDU, the AM RLC entity shall:

-	for the RLC SDUs with SN such that RX_Next <= SN < RX_Highest_Status that has not been completely received yet, in increasing SN order of RLC SDUs and increasing byte segment order within RLC SDUs, starting with SN = RX_Next up to the point where the resulting STATUS PDU still fits to the total size of RLC PDU(s) indicated by lower layer:

-	for an RLC SDU for which no byte segments have been received yet:

-	include in the STATUS PDU a NACK_SN which is set to the SN of the RLC SDU.

-	for a continuous sequence of byte segments of a partly received RLC SDU that have not been received yet:

-	include in the STATUS PDU a set of NACK_SN, SOstart and SOend.

-	for a continuous sequence of RLC SDUs that have not been received yet:

-	include in the STATUS PDU a set of NACK_SN and NACK range;

-	include in the STATUS PDU, if required, a pair of SOstart and SOend.

-	set the ACK_SN to the SN of the next not received RLC SDU which is not indicated as missing in the resulting STATUS PDU.

7.1.2.3.7.3	Test description

7.1.2.3.7.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.2.1.1 with the exception that the AM DRB is configured according to Table 7.1.2.3.7.3.1-1.

Table 7.1.2.3.7.3.1-1: RLC parameters

| t-Reassembly | ms150 |
| --- | --- |
| t-StatusProhibit | ms300 |
| t-PollRetransmit | ms500 |

7.1.2.3.7.3.2	Test procedure sequence

Table 7.1.2.3.7.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | The SS ignores scheduling requests and does not allocate any uplink grant. | - | - | - | - |
| 1 | The SS transmits 4 AMD PDUs with SN=0, 1, 2, and 4. The SS sets the P field of all the AMD PDUs to 0. A time spacing of 20 ms is applied. Record time TA when the AMD PDU with SN=4 is sent. (Note 6) | <-- | AMD PDU (SN=0, P=0) AMD PDU (SN=1, P=0) AMD PDU (SN=2, P=0) AMD PDU (SN=4, P=0) | - | - |
| 2 | The SS waits for 70 ms after the transmission of the first AMD PDU to ensure UE RLC has all the required SDUs available and then assigns 3 UL grants (UL grant allocation type 2) with a time spacing of 20 ms of size 848 bits (UL Grant Allocation type 2). (Note 1) | <-- | (UL grants, 848 bits) | - | - |
| 3 | The UE transmits RLC SDU#1. | --> | (RLC SDU#1) | - | - |
| 4 | The UE transmits RLC SDU#2. | --> | (RLC SDU#2) | - | - |
| 5 | The UE transmits RLC SDU#3. | --> | (RLC SDU#3) | - | - |
| 6 | 60 ms after step 5, the SS transmits a STATUS PDU | <-- | STATUS PDU | - | - |
| 7 | 80 ms after step 5, the SS starts the UL default grant transmission. | - | - | - | - |
| 8 | Check 1: Does the UE transmit a Status Report with NACK_SN=3 and ACK_SN=5? Record time TB (Note 5) Check 2: (TB – TA) = t-Reassembly? | --> | STATUS PDU | 1 | P |
| 9 | 100 ms after the Status Report is received at Step 8, the SS transmits 4 AMD PDUs with SN=5, 6, 8 and 9. The SS sets the P field of all the AMD PDUs to 0. A time spacing of 20 ms is applied. (Note 6) | <-- | AMD PDU (SN=5, P=0) AMD PDU (SN=6, P=0) AMD PDU (SN=8, P=0) AMD PDU (SN=9, P=0) | - | - |
| 10 | Check 1: Does the UE transmit a Status Report with NACK_SN=3 and ACK_SN=7? Record time TC Check 2: (TC – TB) = t-StatusProhibit? | --> | STATUS PDU | 2 | P |
| 11 | The SS ignores scheduling requests unless otherwise specified and does not allocate any uplink grant. | - | - | - | - |
| 12 | 200 ms after step 10, the SS transmits 2 AMD PDUs with SN=3, SN=7. The SS sets the P field of all the AMD PDUs to 0 except for that of the AMD PDU with SN=7. A time spacing of 20 ms is applied. | <-- | AMD PDU (SN=3, P=0) AMD PDU (SN=7, P=1) | - | - |
| 13 | The SS waits for 100 ms after the transmission of the last AMD PDU to ensure UE RLC has all the required SDUs available and then assigns 1 UL grant (UL grant allocation type 3) of size 88 bits. (Note 2) | <-- | (UL grant, 88 bits) | - | - |
| 14 | Check: Does the UE transmit a Status Report with no NACK_SN and ACK_SN = 10? | --> | STATUS PDU | 3 | P |
| 15 | In the second frame following the one scheduled in step 13 the SS assigns 7 UL grants (UL grant allocation type 2) with a time spacing of 20 ms of size 848 bits. (Note 1) | <-- | (UL grant, 848 bits) | - | - |
| 16 | The UE transmits RLC SDU#4. | --> | (RLC SDU#4) | - | - |
| 17 | The UE transmits RLC SDU#5. | --> | (RLC SDU#5) | - | - |
| 18 | The UE transmits RLC SDU#6. | --> | (RLC SDU#6) | - | - |
| 19 | The UE transmits RLC SDU#7. | --> | (RLC SDU#7) | - | - |
| 20 | The UE transmits RLC SDU#8. | --> | (RLC SDU#8) | - | - |
| 21 | The UE transmits RLC SDU#9. | --> | (RLC SDU#9) | - | - |
| 22 | The UE transmits RLC SDU#10. | --> | (RLC SDU#10) | - | - |
| 23 | The SS transmits a STATUS PDU | <-- | STATUS PDU | - | - |
| 24 | After 300 ms the SS transmits an AMD PDU with SN=11 and P=0, and an AMD PDU with SN=12 and P=1.A time spacing of 20 ms is applied. (Note 6) | <-- | AMD PDU (SN=11, P=0) AMD PDU (SN=12, P=1) | - | - |
| 25 | Check: Does the UE transmit a scheduling request within t-Reassembly/ 2 ms after the transmission of the first AMD PDU of Step 24? | --> | (SR) | 4 | F |
| 26 | At t-Reassembly/ 2 ms after the transmission of the second AMD PDU of Step 24, the SS transmits an AMD PDU with SN=10 and P=0. (Note 6) | <-- | AMD PDU (SN=10, P=0) | - | - |
| 27 | The SS waits for 60 ms to ensure UE RLC has all the required SDUs available and then assigns 1 UL grant (UL grant allocation type 3) of size 88 bits. (Note 2) | <-- | (UL grant, 88 bits) | - | - |
| 28 | Check: Does the UE transmit a Status Report with no NACK_SN and ACK_SN=13? | --> | STATUS PDU | 4 | P |
| 29 | The SS assigns 3 UL grants (UL grant allocation type 2) with a time spacing of 20 ms of size 848 bits. (Note 1) | <-- | (UL grant, 848 bits) | - | - |
| 30 | The UE transmits RLC SDU#11. | --> | (RLC SDU#11) | - | - |
| 31 | The UE transmits RLC SDU#12. | --> | (RLC SDU#12) | - | - |
| 32 | The UE transmits RLC SDU#13. | --> | (RLC SDU#13) | - | - |
| 33 | The SS transmits a STATUS PDU. | <-- | STATUS PDU | - | - |
| 34 | After 300 ms the SS transmits an AMD PDU with SN=17 and P=0, and an AMD PDU with SN=19 and P=1. A time spacing of 20 ms is applied. (Note 6) | <-- | AMD PDU (SN=17, P=0) AMD PDU (SN=19, P=1) | - | - |
| 35 | The SS waits for t-Reassembly ms to ensure expiry. | - | - | - | - |
| 36 | 60 ms after step 35 the SS assigns an UL grant (UL grant allocation type 3) of size 88 bits. (Note 3) | <-- | (UL Grant, 88 bits) | - | - |
| 37 | Check: Does the UE transmit a Status Report with ACK_SN=18 and NACK_SN: 13 including NACK Range 4 (SN 13, 14, 15, 16)? | --> | STATUS PDU | 5,6 | P |
| 38 | After 300 ms the SS transmits an AMD PDU with SN=16 and P=1. (Note 6) | <-- | AMD PDU (SN=16, P=1) | - | - |
| 39 | 30 ms after step 38 the SS assigns an UL grant (UL grant allocation type 3) of size 112 bits. (Note 4) | <-- | (UL Grant, size 112) | - | - |
| 40 | Check: Does the UE transmit a Status Report with ACK_SN=20 and NACK_SN: 13 including NACK Range 3 (SN 13, 14, 15) and NACK_SN=18 without NACK Range? | --> | STATUS PDU | 5,6 | P |
| 41 | 60 ms after step 38 the SS transmits 4 AMD PDUs with SN=13, 14, 15 and 18. A time spacing of 20 ms is applied. (Note 6) | <-- | AMD PDU (SN=13, P=0) AMD PDU (SN=14, P=0) AMD PDU (SN=15, P=0) AMD PDU (SN=18, P=0) | - | - |
| 42 | 130 ms after the transmission of the first AMD PDU the SS assigns 7 UL grant (UL grant allocation type 3 with a time spacing of 20 ms of size 848 bits. (Note 1) | <-- | (UL grant, 848 bits) | - | - |
| 43 | The UE loopbacks the complete RLC SDU. | --> | (RLC SDU#14) | - | - |
| 44 | The UE loopbacks the complete RLC SDU. | --> | (RLC SDU#15) | - | - |
| 45 | The UE loopbacks the complete RLC SDU. | --> | (RLC SDU#16) | - | - |
| 46 | The UE loopbacks the complete RLC SDU. | --> | (RLC SDU#17) | - | - |
| 47 | The UE loopbacks the complete RLC SDU. | --> | (RLC SDU#18) | - | - |
| 48 | The UE loopbacks the complete RLC SDU. | --> | (RLC SDU#19) | - | - |
| 49 | The UE loopbacks the complete RLC SDU. | --> | (RLC SDU#20) | - | - |
| 50 | The SS transmits a STATUS PDU. | <-- | STATUS PDU | - | - |
| Note 1: UL grant of 848 bits (LRBs & IMCS as per 38.523-3[3] annex B) is chosen to allow the UE to transmit one PDU at a time ( 99 bytes RLC SDU + 3 bytes RLC Header + 2 bytes MAC Sub PDU header + 2 bytes for short BSR or padding) for (pc_supportOfRedCap_r17=false AND pc_supportOfERedCap_r18= false), or ( 100 bytes RLC SDU + 2 bytes RLC Header + 2 bytes MAC Sub PDU header + 2 bytes for short BSR or padding) for (pc_supportOfRedCap_r17=true OR pc_supportOfERedCap_r18=true). Note 2: UL grant of 88 bits (LRBs & IMCS as per 38.523-3[3] annex B) is chosen to allow the UE to transmit a Status Report with ACK_SN(3 byte) + 2 byte MAC PDU subheader and (2 byte short BSR). 4 Bytes additional space provided to confirm UE does not include NACK_SN and conformant UE instead will include MAC Padding. Note 3: UL grant of 88 bits (LRBs & IMCS as per 38.523-3[3] annex B) is chosen to allow the UE to transmit (a Status Report with ACK_SN (3 Bytes)and 1 NACK_SNs with NACK Range(4 Bytes) + MAC PDU subheader (2 Bytes) + Short BSR (2 Byte)) for (pc_supportOfRedCap_r17=false AND pc_supportOfERedCap_r18= false), or (a Status Report with ACK_SN (3 Bytes) and 1 NACK_SNs with NACK Range (3 Bytes) + MAC PDU subheader (2 Bytes) + Short BSR (2 Bytes) + Padding (1 Bytes)) for (pc_supportOfRedCap_r17=true OR pc_supportOfERedCap_r18=true). Note 4: UL grant of 112 bits (LRBs & IMCS as per 38.523-3[3] annex B) is chosen to allow the UE to transmit (a Status Report with ACK_SN (3 Bytes)and 1 NACK_SNs with NACK Range(4 Bytes) +NACK SN (3 Bytes) + MAC PDU subheader (2 Bytes) + Short BSR (2 Byte)) for (pc_supportOfRedCap_r17=false AND pc_supportOfERedCap_r18= false),or (a Status Report with ACK_SN (3 Bytes) and 1 NACK_SNs with NACK Range (3 Bytes) +NACK SN (2 Bytes) + MAC PDU subheader (2 Bytes) + Short BSR (2 Bytes) + Padding (2 Bytes)) for (pc_supportOfRedCap_r17=true OR pc_supportOfERedCap_r18=true). Note 5: Time TB should be recorded by factoring in the SR-Periodicity as configured in the SchedulingRequestResourceConfig->periodicityAndOffset. Note 6: Every DL AMD PDU contains 1 RLC SDU size of 99 bytes for (pc_supportOfRedCap_r17=false AND pc_supportOfERedCap_r18= false), or 100 bytes for (pc_supportOfRedCap_r17=true OR pc_supportOfERedCap_r18=true). |  |  |  |  |  |

7.1.2.3.7.3.3	Specific message contents

None

##### 7.1.2.3.8	AM RLC / Reconfiguration of RLC parameters by upper layers

7.1.2.3.8.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and using AM RLC }

ensure that {

when { t-PollRetransmit value is changed during reconfiguration of RLC parameters by upper layers}

then { UE starts using new t-PollRetransmit value }

}

(2)

with { UE in RRC_CONNECTED state and using AM RLC }

ensure that {

when { t-Reassembly value is changed during reconfiguration of RLC parameters by upper layers }

then { UE starts using new t-Reassembly value }

}

(3)

with { UE in RRC_CONNECTED state and using AM RLC }

ensure that {

when { t-StatusProhibit value is changed during reconfiguration of RLC parameters by upper layers }

then { UE starts using new t-StatusProhibit value }

}

7.1.2.3.8.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.322, clauses 5.3.3.1, 5.3.3.2, 5.3.3.3, 5.3.4 and 7.3. TS 38.331 clause 5.3.5.5.4. Unless otherwise stated these are Rel-15 requirements.

[TS 38.322, clause 5.3.3.1]

An AM RLC entity can poll its peer AM RLC entity in order to trigger STATUS reporting at the peer AM RLC entity.

[TS 38.322, clause 5.3.3.2]

Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission such that the AMD PDU contains either a not previously transmitted RLC SDU or an RLC SDU segment containing not previously transmitted byte segment, the transmitting side of an AM RLC entity shall:

-	increment PDU_WITHOUT_POLL by one;

-	increment BYTE_WITHOUT_POLL by every new byte of Data field element that it maps to the Data field of the AMD PDU;

-	if PDU_WITHOUT_POLL >= pollPDU; or

-	if BYTE_WITHOUT_POLL >= pollByte:

-	include a poll in the AMD PDU as described below.

Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission, the transmitting side of an AM RLC entity shall:

-	if both the transmission buffer and the retransmission buffer becomes empty (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) after the transmission of the AMD PDU; or

-	if no new RLC SDU can be transmitted after the transmission of the AMD PDU (e.g. due to window stalling);

-	include a poll in the AMD PDU as described below.

NOTE:	Empty RLC buffer (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) should not lead to unnecessary polling when data awaits in the upper layer. Details are left up to UE implementation.

To include a poll in an AMD PDU, the transmitting side of an AM RLC entity shall:

-	set the P field of the AMD PDU to "1";

-	set PDU_WITHOUT_POLL to 0;

-	set BYTE_WITHOUT_POLL to 0.

After submitting an AMD PDU including a poll to lower layer and after incrementing of TX_Next if necessary, the transmitting side of an AM RLC entity shall:

-	set POLL_SN to TX_Next – 1;

-	if t-PollRetransmit is not running:

-	start t-PollRetransmit.

-	else:

-	restart t-PollRetransmit.

[TS 38.322, clause 5.3.3.3]

Upon reception of a STATUS report from the receiving RLC AM entity the transmitting side of an AM RLC entity shall:

-	if the STATUS report comprises a positive or negative acknowledgement for the RLC SDU with sequence number equal to POLL_SN:

-	if t-PollRetransmit is running:

-	stop and reset t-PollRetransmit.

[TS 38.322, clause 5.3.4]

Upon expiry of t-PollRetransmit, the transmitting side of an AM RLC entity shall:

-	if both the transmission buffer and the retransmission buffer are empty (excluding transmitted RLC SDU or RLC SDU segment awaiting acknowledgements); or

-	if no new RLC SDU or RLC SDU segment can be transmitted (e.g. due to window stalling):

-	consider the RLC SDU with SN = TX_Next – 1 for retransmission; or

-	consider any RLC SDU which has not been positively acknowledged for retransmission.

-	include a poll in an AMD PDU as described in section 5.3.3.2.

[TS 38.322, clause 7.3]

The following timers are configured by 3GPP TS 38.331 [5]:

a) t-PollRetransmit

This timer is used by the transmitting side of an AM RLC entity in order to retransmit a poll (see sub clause 5.3.3).

b) t-Reassembly

This timer is used by the receiving side of an AM RLC entity and receiving UM RLC entity in order to detect loss of RLC PDUs at lower layer (see sub clauses 5.2.2.2 and 5.2.3.2). If t-Reassembly is running, t-Reassembly shall not be started additionally, i.e. only one t-Reassembly per RLC entity is running at a given time.

c) t-StatusProhibit

This timer is used by the receiving side of an AM RLC entity in order to prohibit transmission of a STATUS PDU (see sub clause 5.3.4).

[TS 38.331, clause 5.3.5.5.4]

For each RLC-Bearer-Config received in the rlc-BearerToAddModList IE the UE shall:

1>	if the UE’s current configuration contains a RLC bearer with the received logicalChannelIdentity:

2>	if reestablishRLC is received:

3>	re-establish the RLC entity as specified in TS 38.322 [4];

2>	reconfigure the RLC entity or entities in accordance with the received rlc-Config;

2>	reconfigure the logical channel in accordance with the received mac-LogicalChannelConfig;

NOTE:	The network does not re-associate an already configured logical channel with another radio bearer. Hence servedRadioBearer is not present in this case.

7.1.2.3.8.3	Test description

7.1.2.3.8.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.2.1.1 with the exception that the AM DRB is configured according to Table 7.1.2.3.8.3.1-1.

Table 7.1.2.3.8.3.1-1: RLC parameters

| Parameter | Value |
| --- | --- |
| t-Reassembly | ms150 |
| t-StatusProhibit | ms300 |
| t-PollRetransmit | ms400 |
| pollPDU | infinity |
| pollByte | infinity |

7.1.2.3.8.3.2	Test procedure sequence

Table 7.1.2.3.8.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| 1-29 | Same expected sequence as in Table 7.1.2.3.8.3.2-2 with (X=0, t-Reassembly = ms150, t-StatusProhibit = ms300, t-PollRetransmit=ms400) Note 1. | - | - | 1,2,3 | - |
| 30 | The SS transmits NR RRCReconfiguratioin message to reconfigure RLC in the UE and set: - t-Reassembly to ms200, - t-StatusProhibit to ms400, - t-PollRetransmit to ms500. (Note 1) | <-- | RRCReconfiguration | - | - |
| 30A | The UE transmits a NR RRCReconfigurationcomplete message. (Note 2) | --> | RRCReconfigurationComplete | - | - |
| 31-59 | Same expected sequence as in Table 7.1.2.3.8.3.2-2 with (X=11, t-Reassembly = ms200, t-StatusProhibit = ms400, t-PollRetransmit=ms500). | - | - | 1,2,3 | - |
| Note 1: For EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration 36.508 [7], Table 4.6.1-8 using condition EN-DC_EmbedNR_RRCRecon. Note 2: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. |  |  |  |  |  |

Table 7.1.2.3.8.3.2-2: Behaviour Sequence (X, t-Reassembly, t-StatusProhibit,t-PollRetransmit)

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| 1 | The SS ignores scheduling requests and does not allocate any uplink grant. | - | - | - | - |
| 2 | The SS transmits 4 AMD PDUs with P=0 and SN=X, X+1, X+2 and X+4. The SS record time TA when AMD PDU#5 (with SN=X+4) is sent. A time spacing of 20 ms is applied. | <-- | AMD PDU#1 (SN=X, P=0) AMD PDU#2 (SN=X+1, P=0) AMD PDU#3 (SN=X+2, P=0) AMD PDU#5 (SN=X+4, P=0) | - | - |
| 3 | The SS waits for 70 ms after the transmission of the first AMD PDU to ensure UE RLC has all the required SDUs available and then assigns 3 UL grants of size 848 bits with a time spacing of 20 ms. (Note 1) | <-- | (UL grants, 848 bits) | - | - |
| 4 | The UE transmits RLC SDU#1+X. | --> | (RLC SDU#1+X) | - | - |
| 5 | The UE transmits RLC SDU#2+X. | --> | (RLC SDU#2+X) | - | - |
| 6 | The UE transmits RLC SDU#3+X. | --> | (RLC SDU#3+X) | - | - |
| 7 | 60 ms after step 3 the SS transmits a STATUS PDU. | <-- | STATUS PDU | - | - |
| 8 | The SS starts the UL default grant transmission. | - | - | - | - |
| 9 | Check 1: Does the UE transmit a STATUS PDU with NACK_SN=X+3 and ACK_SN=X+5? Record time TB (Note 5). Check 2: Is (TB – TA ) = t-Reassembly? | --> | STATUS PDU | 2 | P |
| 10 | 100 ms after the Status Report received at Step 9, the SS sends 4 AMD PDUs with P=0 and SN=X+5, X+6, X+8 and X+9. A time spacing of 20 ms is applied. | <-- | AMD PDU#6 (SN=X+5, P=0) AMD PDU#7 (SN=X+6, P=0) AMD PDU#9 (SN=X+8, P=0) AMD PDU#10 (SN=X+9, P=0) | - | - |
| 11 | Check 1: Does the UE transmit a Status Report with NACK_SN=X+3 and ACK_SN=X+7? Record time TC Check 2: (TC – TB) = t-StatusProhibit? | --> | STATUS PDU | 3 | P |
| 12 | The SS ignores scheduling requests and does not allocate any uplink grant. | - | - | - | - |
| 13 | After 250 ms the SS transmits 3 AMD PDUs with SN=X+3, X+7 and X+9. The SS sets the P field of all the AMD PDUs to 0 except for that of the AMD PDU with SN=X+9. A time spacing of 20 ms is applied. | <-- | AMD PDU#4 (SN=X+3, P=0) AMD PDU#8 (SN=X+7, P=0) AMD PDU#10 (SN=X+9, P=1) | - | - |
| 14 | The SS waits for 150 ms after AMD PDU#4 transmission to ensure UE RLC has all the required SDUs available and then assigns 1 UL grant of size 88 bits (UL Grant Allocation type 3). (Note 2)(Note 4) | <-- | (UL grant, 88 bits) | - | - |
| 15 | The UE transmits a Status Report with no NACK_SN and ACK_SN=X+10. | --> | STATUS PDU | - | - |
| 16 | In the subframe following the one scheduled in step 14 the SS assigns 7 UL grants of size 848 bits (UL Grant Allocation type 2) with a time spacing of 20 ms. (Note 1) | <-- | (UL grants, 848 bits) | - | - |
| 17 | The UE transmits RLC SDU#4+X. | --> | (RLC SDU#4+X) | - | - |
| 18 | The UE transmits RLC SDU#5+X. | --> | (RLC SDU#5+X) | - | - |
| 19 | The UE transmits RLC SDU#6+X. | --> | (RLC SDU#6+X) | - | - |
| 20 | The UE transmits RLC SDU#7+X. | --> | (RLC SDU#7+X) | - | - |
| 21 | The UE transmits RLC SDU#8+X. | --> | (RLC SDU#8+X) | - | - |
| 22 | The UE transmits RLC SDU#9+X. | --> | (RLC SDU#9+X) | - | - |
| 23 | The UE transmits RLC SDU#10+X. | --> | (RLC SDU#10+X) | - | - |
| 24 | The SS transmits a STATUS PDU. | <-- | STATUS PDU | - | - |
| 25 | The SS transmits an AMD PDU to the UE. | <-- | AMD PDU#11 (SN=X+10, P=0) | - | - |
| 26 | The SS starts the UL default grant transmission. | - | - | - | - |
| 27 | The UE transmits an AMD PDU with the same data as received in the corresponding DL AMD PDU. Record time TD. | --> | AMD PDU#11 (SN=X+10, P=1) | - | - |
| 28 | Check 1: Does the UE set the poll bit as both the transmission and retransmission buffers become empty? Record time TE. Check 2: Is (TE – TD ) = t-PollRetransmit? | --> | AMD PDU#11 (SN=X+10, P=1) | 1 | P |
| 29 | The SS transmits a STATUS PDU | <-- | STATUS PDU | - | - |
| Note 1: UL grant of 848 bits (LRBs & IMCS as per 38.523-3[3] annex B) is chosen to allow the UE to transmit one PDU ((99 bytes RLC SDU + 3 bytes RLC Header + 2 bytes MAC Sub PDU header + 2 bytes for short BSR or padding) for (pc_supportOfRedCap_r17=false AND pc_supportOfERedCap_r18= false) or (100 bytes RLC SDU + 2 bytes RLC Header + 2 bytes MAC Sub PDU header + 2 bytes for short BSR or padding) for (pc_supportOfRedCap_r17=true OR pc_supportOfERedCap_r18=true)) at a time. Note 2: UL grant of 88 bits (LRBs & IMCS as per 38.523-3[3] annex B) is chosen to allow the UE to transmit a Status Report with ACK_SN(3 byte) + 2 byte MAC PDU subheader and (2 byte short BSR). 4 Bytes additional space provided to confirm UE does not include NACK_SN and conformant UE instead will include MAC Padding. Note 3: Every DL AMD PDU contains 1 RLC SDU size of 99 bytes for (pc_supportOfRedCap_r17=false AND pc_supportOfERedCap_r18= false) or 100 bytes for (pc_supportOfRedCap_r17=true OR pc_supportOfERedCap_r18=true). Note 4: Timing difference between step 11 to step 15 is equal to t-statusProhibit timer. UE starts SR from step 13 with AMD-PDU#4. 150 ms is chosen so that UE does not reach sr-transMax(n16) otherwise UE RACHes. Note 5: Time TB should be recorded by factoring in the SR-Periodicity as configured in the SchedulingRequestResourceConfig->periodicityAndOffset. |  |  |  |  |  |

7.1.2.3.8.3.3	Specific message contents

Table 7.1.2.3.8.3.3-1: RRCReconfiguration (step 30, Table 7.1.2.3.8.3.2-1)

| Derivation Path: 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| secondaryCellGroup | CellGroupConfig |  | EN-DC |
| nonCriticalExtension SEQUENCE { |  |  | NR |
| masterCellGroup | CellGroupConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.2.3.8.3.3-2: CellGroupConfig (Table 7.1.2.3.8.3.3-1)

| Derivation Path: 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 1 entry |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig | entry 1 |  |
| } |  |  |  |
| mac-CellGroupConfig | Not present |  |  |
| physicalCellGroupConfig | Not present |  |  |
| spCellConfig | Not present |  |  |
| } |  |  |  |

Table 7.1.2.2.8.3.3-3: RLC-BearerConfig (Table 7.1.2.3.8.3.3-2)

| Derivation Path: 38.508-1 [4], Table 4.6.3-148 with condition AM |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-BearerConfig ::= SEQUENCE { |  |  |  |
| logicalChannelIdentity | Set to LCID of the DRB under test |  |  |
| rlc-Config | RLC-Config |  |  |
| } |  |  |  |

Table 7.1.2.2.8.3.3-4: RLC-Config (Table 7.1.2.3.8.3.3-3)

| Derivation Path: 38.508-1 [4], Table 4.6.3-149 with condition AM |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-Config ::= CHOICE { |  |  |  |
| am SEQUENCE { |  |  |  |
| ul-AM-RLC SEQUENCE { |  |  |  |
| sn-FieldLength | Not present |  |  |
| t-PollRetransmit | ms500 |  |  |
| } |  |  |  |
| dl-AM-RLC SEQUENCE { |  |  |  |
| sn-FieldLength | Not present |  |  |
| t-Reassembly | ms200 |  |  |
| t-StatusProhibit | ms400 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 7.1.2.3.9	AM RLC / Reassembling of AMD PDUs

7.1.2.3.9.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives AMD PDUs，and all bytes of the RLC SDU(s) with SN = x are received }

then { UE reassembles the RLC SDU(s) from AMD PDU(s) with SN = x }

}

(2)

with { UE in RRC_CONNECTED state }

ensure that {

when { t-Reassembly expires }

then { update RX_Highest_Status to the SN of the first RLC SDU with SN >= RX_Next_Status_Trigger for which not all bytes have been received }

}

(3)

with { UE in RRC_CONNECTED state }

ensure that {
  when { UE receives AM PDU segments }

then { UE delivers reassembled RLC SDU to upper layer }

}

(4)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives duplicate RLC AM PDU segments }

then { UE discards duplicate RLC AMD PDU segments }

}

(5)

with { UE in  RRC_CONNECTED state }

ensure that {

when { UE receives RLC AMD PDU segments with segments lost }

then { UE transmits STATUS PDU to request retransmission of missing segments }

}

(6)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives overlapping RLC AMD PDU segments }

then { UE discards duplicate RLC AMD PDU byte segments }

}

(7)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives an AMD PDU with a SN gap }

then { UE sends STATUS PDU to request retransmissions of PDUs in the SN gap}

}

7.1.2.3.9.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 38.322 clauses 4.2.1.3.3, 5.2.3.2.1, 5.2.3.2.2, 5.2.3.2.3, 5.2.3.2.4 and 5.3.4. Unless otherwise stated these are Rel-15 requirements.

[TS 38.322, clause 4.2.1.3.3]

When the receiving side of an AM RLC entity receives AMD PDUs, it shall:

-	detect whether or not the AMD PDUs have been received in duplication, and discard duplicated AMD PDUs;

-	detect the loss of AMD PDUs at lower layers and request retransmissions to its peer AM RLC entity;

-	reassemble RLC SDUs from the received AMD PDUs and deliver the RLC SDUs to upper layer as soon as they are available.

[TS 38.322, clause 5.2.3.2.1]

The receiving side of an AM RLC entity shall maintain a receiving window according to the state variable RX_Next as follows:

-	a SN falls within the receiving window if RX_Next <= SN < RX_Next + AM_Window_Size;

-	a SN falls outside of the receiving window otherwise.

When receiving an AMD PDU from lower layer, the receiving side of an AM RLC entity shall:

-	either discard the received AMD PDU or place it in the reception buffer (see sub clause 5.2.3.2.2);

-	if the received AMD PDU was placed in the reception buffer:

-	update state variables, reassemble and deliver RLC SDUs to upper layer and start/stop t-Reassembly as needed (see sub clause 5.2.3.2.3).

When t-Reassembly expires, the receiving side of an AM RLC entity shall:

-	update state variables and start t-Reassembly as needed (see sub clause 5.2.3.2.4).

[TS 38.322, clause 5.2.3.2.2]

When an AMD PDU is received from lower layer, where the AMD PDU contains byte segment numbers y to z of an RLC SDU with SN = x, the receiving side of an AM RLC entity shall:

-	if x falls outside of the receiving window; or

-	if byte segment numbers y to z of the RLC SDU with SN = x have been received before:

-	discard the received AMD PDU.

-	else:

-	place the received AMD PDU in the reception buffer;

-	if some byte segments of the RLC SDU contained in the AMD PDU have been received before:

-	discard the duplicate byte segments.

[TS 38.322, clause 5.2.3.2.3]

When an AMD PDU with SN = x is placed in the reception buffer, the receiving side of an AM RLC entity shall:

-	if x >= RX_Next_Highest

-	update RX_Next_Highest to x+ 1.

-	if all bytes of the RLC SDU with SN = x are received:

-	reassemble the RLC SDU from AMD PDU(s) with SN = x, remove RLC headers when doing so and deliver the reassembled RLC SDU to upper layer;

-	if x = RX_Highest_Status,

-	update RX_Highest_Status to the SN of the first RLC SDU with SN > current RX_Highest_Status for which not all bytes have been received.

-	if x = RX_Next:

-	update RX_Next to the SN of the first RLC SDU with SN > current RX_Next for which not all bytes have been received.

-	if t-Reassembly is running:

-	if RX_Next_Status_Trigger = RX_Next; or

-	if RX_Next_Status_Trigger = RX_Next + 1 and there is no missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU; or

-	if RX_Next_Status_Trigger falls outside of the receiving window and RX_Next_Status_Trigger is not equal to RX_Next + AM_Window_Size:

-	stop and reset t-Reassembly.

-	if t-Reassembly is not running (includes the case t-Reassembly is stopped due to actions above):

-	if RX_Next_Highest> RX_Next +1; or

-	if RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU:

-	start t-Reassembly;

-	set RX_Next_Status_Trigger to RX_Next_Highest.

[TS 38.322, clause 5.2.3.2.4]

When t-Reassembly expires, the receiving side of an AM RLC entity shall:

-	update RX_Highest_Status to the SN of the first RLC SDU with SN >= RX_Next_Status_Trigger for which not all bytes have been received;

-	if RX_Next_Highest> RX_Highest_Status +1: or

-	if RX_Next_Highest = RX_Highest_Status + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Highest_Status before the last byte of all received segments of this SDU:

-	start t-Reassembly;

-	set RX_Next_Status_Trigger to RX_Next_Highest.

[TS 38.322, clause 5.3.4]

An AM RLC entity sends STATUS PDUs to its peer AM RLC entity in order to provide positive and/or negative acknowledgements of RLC SDUs (or portions of them).

Triggers to initiate STATUS reporting include:

-	Polling from its peer AM RLC entity:

-	When an AMD PDU with SN = x and the P field set to "1" is received from lower layer, the receiving side of an AM RLC entity shall:

-	if the AMD PDU is to be discarded as specified in subclause 5.2.3.2.2; or

-	if x < RX_Highest_Status or x >= RX_Next + AM_Window_Size:

-	trigger a STATUS report.

-	else:

-	delay triggering the STATUS report until x < RX_Highest_Status or x >= RX_Next + AM_Window_Size.

NOTE 1:	This ensures that the RLC Status report is transmitted after HARQ reordering.

-	Detection of reception failure of an AMD PDU

-	The receiving side of an AM RLC entity shall trigger a STATUS report when t-Reassembly expires.

NOTE 2:	The expiry of t-Reassembly triggers both RX_Highest_Status to be updated and a STATUS report to be triggered, but the STATUS report shall be triggered after RX_Highest_Status is updated.

When STATUS reporting has been triggered, the receiving side of an AM RLC entity shall:

-	if t-StatusProhibit is not running:

-	at the first transmission opportunity indicated by lower layer, construct a STATUS PDU and submit it to lower layer.

-	else:

-	at the first transmission opportunity indicated by lower layer after t-StatusProhibit expires, construct a single STATUS PDU even if status reporting was triggered several times while t-StatusProhibit was running and submit it to lower layer.

When a STATUS PDU has been submitted to lower layer, the receiving side of an AM RLC entity shall:

-	start t-StatusProhibit.

When constructing a STATUS PDU, the AM RLC entity shall:

-	for the RLC SDUs with SN such that RX_Next <= SN < RX_Highest_Status that has not been completely received yet, in increasing SN order of RLC SDUs and increasing byte segment order within RLC SDUs, starting with SN = RX_Next up to the point where the resulting STATUS PDU still fits to the total size of RLC PDU(s) indicated by lower layer:

-	for an RLC SDU for which no byte segments have been received yet:

-	include in the STATUS PDU a NACK_SN which is set to the SN of the RLC SDU.

-	for a continuous sequence of byte segments of a partly received RLC SDU that have not been received yet:

-	include in the STATUS PDU a set of NACK_SN, SOstart and SOend.

-	for a continuous sequence of RLC SDUs that have not been received yet:

-	include in the STATUS PDU a set of NACK_SN and NACK range;

-	include in the STATUS PDU, if required, a pair of SOstart and SOend.

-	set the ACK_SN to the SN of the next not received RLC SDU which is not indicated as missing in the resulting STATUS PDU.

When STATUS reporting has been triggered, the receiving side of an AM RLC entity shall:

-	if t-StatusProhibit is not running:

-	at the first transmission opportunity indicated by lower layer, construct a STATUS PDU and submit it to lower layer.

-	else:

-	at the first transmission opportunity indicated by lower layer after t-StatusProhibit expires, construct a single STATUS PDU even if status reporting was triggered several times while t-StatusProhibit was running and submit it to lower layer.

When a STATUS PDU has been submitted to lower layer, the receiving side of an AM RLC entity shall:

-	start t-StatusProhibit.

When constructing a STATUS PDU, the AM RLC entity shall:

-	for the RLC SDUs with SN such that RX_Next <= SN < RX_Highest_Status that has not been completely received yet, in increasing SN order of RLC SDUs and increasing byte segment order within RLC SDUs, starting with SN = RX_Next up to the point where the resulting STATUS PDU still fits to the total size of RLC PDU(s) indicated by lower layer:

-	for an RLC SDU for which no byte segments have been received yet:

-	include in the STATUS PDU a NACK_SN which is set to the SN of the RLC SDU.

-	for a continuous sequence of byte segments of a partly received RLC SDU that have not been received yet:

-	include in the STATUS PDU a set of NACK_SN, SOstart and SOend.

-	for a continuous sequence of RLC SDUs that have not been received yet:

-	include in the STATUS PDU a set of NACK_SN and NACK range;

-	include in the STATUS PDU, if required, a pair of SOstart and SOend.

-	set the ACK_SN to the SN of the next not received RLC SDU which is not indicated as missing in the resulting STATUS PDU.

7.1.2.3.9.3	Test description

7.1.2.3.9.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.2.1.1 with the exception that the AM DRB is configured according to Table 7.1.2.3.9.3.1-1.

Table 7.1.2.3.9.3.1-1: RLC parameters

| Parameter | Value |
| --- | --- |
| t-Reassembly | ms150 |
| t-StatusProhibit | ms300 |
| t-PollRetransmit | ms500 |

7.1.2.3.9.3.2	Test procedure sequence

Table 7.1.2.3.9.3.2-1: Main behaviour

| St | Procedure |  | Message Sequence | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| 0 | The SS stops the UL grant transmission. | - | - | - | - |
| 1 | The SS transmits AMD PDU#1 containing a complete RLC SDU#4 (89 bytes and SI field=00). (Note 4) | <-- | AMD PDU#1 (SN=3) | - | - |
| 2 | The SS transmits AMD PDU#2 containing the last segment (44 bytes) of RLC SDU#1 (SI field=10, SO=45). | <-- | AMD PDU#2 (SN=0) segment 2 | - | - |
| 3 | The SS transmits AMD PDU#3 containing the last segment (44 bytes) of RLC SDU#2 (SI field=10, SO=45). | <-- | AMD PDU#3 (SN=1) segment 2 | - | - |
| 4 | The SS transmits AMD PDU#4 containing the first segment (45 bytes) of RLC SDU#2 (SI field=01). | <-- | AMD PDU#4 (SN=1) segment 1 | - | - |
| 5 | The SS transmits AMD PDU#5 containing the first segment (45 bytes) of RLC SDU#1 (SI field =01). | <-- | AMD PDU#5 (SN=0) segment 1 | - | - |
| 6 | The SS waits for 20 ms then SS transmits 2 uplink grants with a time spacing of 20 ms. (Note 1) | <-- | UL Grants | - | - |
| 7 | Check: Does the UE transmit an AMD PDU containing RLC SDU#1 in its data field? | --> | AMD PDU (RLC SDU#1) | 1,3 | P |
| 8 | Check: Does the UE transmit an AMD PDU containing RLC SDU#2 in its data field? | --> | AMD PDU (RLC SDU#2) | 1,3 | P |
| 9 | Void |  |  |  |  |
| 9A | 110 ms after step 5 the SS starts the UL default grant transmission. | <-- | UL Grant | - | - |
| 10 | Wait for t-reassembly of UE side to expire. Check: Does the UE transmit an RLC STATUS PDU with NACK_SN=2 and ACK_SN=4 to correctly to inform SS of missing RLC SDU#3? | --> | STATUS PDU (ACK_SN=4, NACK_SN=2) | 2,7 | P |
| 10A | The SS stops the UL grant transmission. | - | - | - | - |
| 11 | 120 ms after step 10 the SS transmits AMD PDU#6 containing the first 45 bytes of SDU#3 in its data field. SO=0 and LSF=0. No header extension part is provided.(Note 5) | <-- | AMD PDU#6 (SN=2) segment 1 | - | - |
| 11A | 20 ms after step 11 the SS transmits AMD PDU#6 containing the first 45 bytes of SDU#3 in its data field. SO=0 and LSF=0. No header extension part is provided. | <-- | AMD PDU#6 (SN=2) segment 1 | - | - |
| 12 | 40 ms after step 11 the SS transmits AMD PDU#12 containing the last 44 bytes of SDU#3 in its data field, with the P-bit set. SO=45 and LSF=1. No header extension part is provided. | <-- | AMD PDU#12 (SN=2, P=1) segment 2 | - | - |
| 13 | After the expiry of t-StatusProhibit timer started at step 10, the SS assigns 1 UL grant (UL grant allocation type 3) of size 88 bits. (Note 2) | <-- | UL Grant | - | - |
| 14 | Check: Does the UE transmit a STATUS PDU with ACK_SN=4, thus acknowledging the reception of PDUs with SN=0 to SN=3, and no NACK_SN provided? | --> | STATUS PDU | 1,3,4 | P |
| 14A | The SS waits for 40 ms to ensure UE has all the required SDUs available and then assigns 2 UL grants of 768 bits (Note 1). | <-- | UL Grant | - | - |
| 15 | Check: Does the UE transmit RLC SDU#3? | --> | (RLC SDU#3) | 1,3,4 | P |
| 15A | Check: Does the UE transmit RLC SDU#4 with the P-bit set? | --> | (RLC SDU#4) | 1,3,4 | P |
| 16 | The SS transmits a STATUS PDU. | <-- | STATUS PDU (ACK SN=4) | - | - |
| 17 | The SS transmits AMD PDU#7 containing the last segment (44 bytes) of RLC SDU#5 (SI field=10, SO=45). | <-- | AMD PDU#7 (SN=4) segment 2 | - | - |
| 17A | The SS starts the UL default grant transmission. | <-- | UL Grant | - | - |
| 18 | Wait for t-reassembly of UE side to expire. Check: Does the UE transmit an RLC STATUS PDU with ACK_SN=5, NACK_SN=4 with SOStart=0 and SOEnd=44? | --> | STATUS PDU (ACK_SN=5, NACK_SN=4 with SOStart=0 /SOEnd=44) | 2,5 | P |
| 18A | The SS stops the UL grant transmission. | - | - | - | - |
| 19 | 160 ms after step 18 The SS transmits AMD PDU#8 containing the first segment (45 bytes) of RLC SDU#5 (SI field=01). (Note 6) | <-- | AMD PDU#8 (SN=4, P=1) segment 1 | - | - |
| 20 | Void |  |  |  |  |
| 21 | Void |  |  |  |  |
| 21A | After the expiry of t-StatusProhibit timer started at step 18, the SS assigns 1 UL grant (UL grant allocation type 3) of size 88 bits. (Note 2) | <-- | UL Grant | - | - |
| 22 | Check: Does the UE transmit an RLC STATUS PDU with ACK_SN=5? | --> | STATUS PDU (ACK_SN=5) | 1,3,5 | P |
| 22A | The SS waits for 100 ms then SS transmits one uplink grant. (Note 1) | <-- | UL Grant | - | - |
| 22B | Check: Does the UE transmit an AMD PDU containing RLC SDU#5 in its data field with the P-bit set? | --> | AMD PDU (RLC SDU#5) | 1,3,5 | P |
| 22C | The SS transmits a STATUS PDU | <-- | STATUS PDU (ACK SN=5) |  |  |
| 23 | The SS transmits AMD PDU#9 containing the last 29 bytes of RLC SDU#6 (SI field=10, SO=60). | <-- | AMD PDU#9 (SN=5) segment 3 | - | - |
| 23A | The SS starts the UL default grant transmission. | <-- | UL Grant | - | - |
| 24 | Wait for t-reassembly of UE side to expire. Check: Does the UE transmit an RLC STATUS PDU with ACK_SN=6, NACK_SN=5 with SOStart=0 and SOEnd=59? | --> | STATUS PDU (ACK_SN=6, NACK_SN=5 with SOStart=0 / SOEnd=59) | 2,5 | P |
| 25 | The SS transmits AMD PDU#10 containing the last 49 byte of RLC SDU#6 (SI field=10, SO=40). | <-- | AMD PDU#10 (SN=5) segment 2 | - | - |
| 25A | Void. |  |  |  |  |
| 26 | Wait for t-reassembly of UE side to expire. Check: Does the UE transmit an RLC STATUS PDU with ACK_SN=6, NACK_SN=5 with SOStart=0 and SOEnd=39? | --> | STATUS PDU (ACK_SN=6, NACK_SN=5 with SOStart=0 / SOEnd=39) | 2,6 | P |
| 26A | The SS stops the UL grant transmission. | - | - | - | - |
| 27 | 160 ms after step 26 The SS transmits AMD PDU#11 containing the first 40 bytes of RLC SDU#6 (SI field =01). | <-- | AMD PDU#11 (SN=5, P=1) segment 1 | - | - |
| 28 | Void |  |  |  |  |
| 29 | Void |  |  |  |  |
| 29A | After the expiry of t-StatusProhibit timer started at step 26, the SS assigns 1 UL grant (UL grant allocation type 3) of size 88 bits. (Note 2, Note 7) | <-- | UL Grant | - | - |
| 30 | Check: Does the UE transmit an RLC STATUS PDU with ACK_SN=6, thus acknowledging the reception of RLC SDUs with SN=0 to SN=5, and no NACK_SN provided? | --> | STATUS PDU (ACK_SN=6) | 1,3,6 | P |
| 30A | The SS transmits one uplink grant. (Note 1) | <-- | UL Grant | - | - |
| 30B | Check: Does the UE transmit an AMD PDU containing RLC SDU#6 in its data field with the P-bit set? | --> | AMD PDU (RLC SDU#6) | 1,3,6 | P |
| 31 | The SS transmits a STATUS PDU. | <-- | STATUS PDU (ACK SN=6) | - | - |
| Note 1: UL grant of 768 bits(LRBs & IMCS as per 38.523-3[3] annex B) is chosen to allow the UE to transmit one PDU at a time( 89 bytes RLC SDU + 3 bytes RLC Header + 2 bytes MAC Sub PDU header + 2 bytes for short BSR or padding) for (pc_supportOfRedCap_r17=false AND pc_supportOfERedCap_r18= false), or one PDU (89 bytes RLC SDU + 2 bytes RLC Header + 2 bytes MAC Sub PDU header + 3 bytes for short BSR and padding or only padding) for (pc_supportOfRedCap_r17=true OR pc_supportOfERedCap_r18=true). Note 2: UL grant of 88 bits (LRBs & IMCS as per 38.523-3 [3] annex B) is chosen to allow the UE to transmit a Status Report with ACK_SN(3 byte) + 2 Bytes MAC PDU subheader and (2 Bytes short BSR). 4 Bytes additional space provided to confirm UE does include resp. does not include NACK_SN and conformant UE instead will include MAC Padding. Note 3: Void Note 4: The PDUs in steps 1-5 are scheduled with a 20 ms time spacing. The UL grant provision in step 6 is scheduled 20 ms later That way this step sequence takes less than t-Reassembly. Note 5: Timing difference between step 10 to step 14 is equal to t-statusProhibit timer. UE starts SR from step 12. 120 ms is chosen so that UE does not reach sr-transMax(n16) otherwise UE RACHes. Note 6: Timing difference between step 18 to step 22 is equal to t-statusProhibit timer. UE starts SR from step 19. 160 ms is chosen so that UE does not reach sr-transMax(n16) otherwise UE RACHes. Note 7: Timing difference between step 26 to step 30 is equal to t-statusProhibit timer. UE starts SR from step 27. 160 ms is chosen so that UE does not reach sr-transMax(n16) otherwise UE RACHes. |  |  |  |  |  |

7.1.2.3.9.3.3	Specific message contents

None.

##### 7.1.2.3.9a	NR NTN / AM RLC / t-Reassembly expiry / t-ReassemblyExt-r17 configured

7.1.2.3.9a.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state on a cell which provides access by NR NTN and using AM RLC }

ensure that {

when { t-ReassemblyExt-r17 is configured }

then { UE ignores value signalled in t-Reassembly and updates RX_Highest_Status to the SN of the first RLC SDU with SN >= RX_Next_Status_Trigger for which not all bytes have been received considering the value signalled in t-ReassemblyExt-r17 }

}

7.1.2.3.9a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.322, clauses 4.2.1.3.3, 5.2.3.2.1, 5.2.3.2.3, 5.2.3.2.4, 5.3.4, 7.1 and TS 38.331, clause 6.3.2. Unless otherwise stated these are Rel-17 requirements.

[TS 38.322, clause 4.2.1.3.3]

When the receiving side of an AM RLC entity receives AMD PDUs, it shall:

-	detect whether or not the AMD PDUs have been received in duplication, and discard duplicated AMD PDUs;

-	detect the loss of AMD PDUs at lower layers and request retransmissions to its peer AM RLC entity;

-	reassemble RLC SDUs from the received AMD PDUs and deliver the RLC SDUs to upper layer as soon as they are available.

[TS 38.322, clause 5.2.3.2.1]

The receiving side of an AM RLC entity shall maintain a receiving window according to the state variable RX_Next as follows:

-	a SN falls within the receiving window if RX_Next <= SN < RX_Next + AM_Window_Size;

-	a SN falls outside of the receiving window otherwise.

When receiving an AMD PDU from lower layer, the receiving side of an AM RLC entity shall:

-	either discard the received AMD PDU or place it in the reception buffer (see clause 5.2.3.2.2);

-	if the received AMD PDU was placed in the reception buffer:

-	update state variables, reassemble and deliver RLC SDUs to upper layer and start/stop t-Reassembly as needed (see clause 5.2.3.2.3).

When t-Reassembly expires, the receiving side of an AM RLC entity shall:

-	update state variables and start t-Reassembly as needed (see clause 5.2.3.2.4).

[TS 38.322, clause 5.2.3.2.3]

When an AMD PDU with SN = x is placed in the reception buffer, the receiving side of an AM RLC entity shall:

-	if x >= RX_Next_Highest:

-	update RX_Next_Highest to x+ 1.

-	if all bytes of the RLC SDU with SN = x are received:

-	reassemble the RLC SDU from AMD PDU(s) with SN = x, remove RLC headers when doing so and deliver the reassembled RLC SDU to upper layer;

-	if x = RX_Highest_Status:

-	update RX_Highest_Status to the SN of the first RLC SDU with SN > current RX_Highest_Status for which not all bytes have been received.

-	if x = RX_Next:

-	update RX_Next to the SN of the first RLC SDU with SN > current RX_Next for which not all bytes have been received.

-	if t-Reassembly is running:

-	if RX_Next_Status_Trigger = RX_Next; or

-	if RX_Next_Status_Trigger = RX_Next + 1 and there is no missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU; or

-	if RX_Next_Status_Trigger falls outside of the receiving window and RX_Next_Status_Trigger is not equal to RX_Next + AM_Window_Size:

-	stop and reset t-Reassembly.

-	if t-Reassembly is not running (includes the case t-Reassembly is stopped due to actions above):

-	if RX_Next_Highest> RX_Next +1; or

-	if RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU:

-	start t-Reassembly;

-	set RX_Next_Status_Trigger to RX_Next_Highest.

[TS 38.322, clause 5.2.3.2.4]

When t-Reassembly expires, the receiving side of an AM RLC entity shall:

-	update RX_Highest_Status to the SN of the first RLC SDU with SN >= RX_Next_Status_Trigger for which not all bytes have been received;

-	if RX_Next_Highest> RX_Highest_Status +1: or

-	if RX_Next_Highest = RX_Highest_Status + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Highest_Status before the last byte of all received segments of this SDU:

-	start t-Reassembly;

-	set RX_Next_Status_Trigger to RX_Next_Highest.

[TS 38.322, clause 5.3.4]

An AM RLC entity sends STATUS PDUs to its peer AM RLC entity in order to provide positive and/or negative acknowledgements of RLC SDUs (or portions of them).

Triggers to initiate STATUS reporting include:

-	Polling from its peer AM RLC entity:

-	When an AMD PDU with SN = x and the P field set to "1" is received from lower layer, the receiving side of an AM RLC entity shall:

-	if the AMD PDU is to be discarded as specified in clause 5.2.3.2.2; or

-	if x < RX_Highest_Status or x >= RX_Next + AM_Window_Size:

-	trigger a STATUS report.

-	else:

-	delay triggering the STATUS report until x < RX_Highest_Status or x >= RX_Next + AM_Window_Size.

NOTE 1:	This ensures that the RLC Status report is transmitted after HARQ reordering.

-	Detection of reception failure of an AMD PDU

-	The receiving side of an AM RLC entity shall trigger a STATUS report when t-Reassembly expires.

NOTE 2:	The expiry of t-Reassembly triggers both RX_Highest_Status to be updated and a STATUS report to be triggered, but the STATUS report shall be triggered after RX_Highest_Status is updated.

When STATUS reporting has been triggered, the receiving side of an AM RLC entity shall:

-	if t-StatusProhibit is not running:

-	at the first transmission opportunity indicated by lower layer, construct a STATUS PDU and submit it to lower layer.

-	else:

-	at the first transmission opportunity indicated by lower layer after t-StatusProhibit expires, construct a single STATUS PDU even if status reporting was triggered several times while t-StatusProhibit was running and submit it to lower layer.

When a STATUS PDU has been submitted to lower layer, the receiving side of an AM RLC entity shall:

-	start t-StatusProhibit.

When constructing a STATUS PDU, the AM RLC entity shall:

-	for the RLC SDUs with SN such that RX_Next <= SN < RX_Highest_Status that has not been completely received yet, in increasing SN order of RLC SDUs and increasing byte segment order within RLC SDUs, starting with SN = RX_Next up to the point where the resulting STATUS PDU still fits to the total size of RLC PDU(s) indicated by lower layer:

-	for an RLC SDU for which no byte segments have been received yet:

-	include in the STATUS PDU a NACK_SN which is set to the SN of the RLC SDU.

-	for a continuous sequence of byte segments of a partly received RLC SDU that have not been received yet:

-	include in the STATUS PDU a set of NACK_SN, SOstart and SOend.

-	for a continuous sequence of RLC SDUs that have not been received yet:

-	include in the STATUS PDU a set of NACK_SN and NACK range;

-	include in the STATUS PDU, if required, a pair of SOstart and SOend.

-	set the ACK_SN to the SN of the next not received RLC SDU which is not indicated as missing in the resulting STATUS PDU.

[TS 38.322, clause 7.1]

This clause describes the state variables used in AM and UM entities in order to specify the RLC protocol. The state variables defined in this clause are normative.

All state variables and all counters are non-negative integers.

All state variables related to AM data transfer can take values from 0 to 4095 for 12 bit SN or from 0 to 262143 for 18 bit SN. All arithmetic operations contained in the present document on state variables related to AM data transfer are affected by the AM modulus (i.e. final value = [value from arithmetic operation] modulo 4096 for 12 bit SN and 262144 for 18 bit SN).

…

When performing arithmetic comparisons of state variables or SN values, a modulus base shall be used.

TX_Next_Ack and RX_Next shall be assumed as the modulus base at the transmitting side and receiving side of an AM RLC entity, respectively. This modulus base is subtracted from all the values involved, and then an absolute comparison is performed (e.g. RX_Next <= SN < RX_Next + AM_Window_Size is evaluated as [RX_Next – RX_Next] modulo 2[sn-FieldLength] <= [SN – RX_Next] modulo 2[sn-FieldLength] < [RX_Next + AM_Window_Size – RX_Next] modulo 2[sn-FieldLength]), where sn-FieldLength is 12 or 18 for 12 bit SN and 18 bit SN, respectively.

…

The transmitting side of each AM RLC entity shall maintain the following state variables:

a) TX_Next_Ack – Acknowledgement state variable

This state variable holds the value of the SN of the next RLC SDU for which a positive acknowledgment is to be received in-sequence, and it serves as the lower edge of the transmitting window. It is initially set to 0, and is updated whenever the AM RLC entity receives a positive acknowledgment for an RLC SDU with SN = TX_Next_Ack.

b) TX_Next – Send state variable

This state variable holds the value of the SN to be assigned for the next newly generated AMD PDU. It is initially set to 0, and is updated whenever the AM RLC entity constructs an AMD PDU with SN = TX_Next and contains an RLC SDU or the last segment of a RLC SDU.

c) POLL_SN – Poll send state variable

This state variable holds the value of the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer when POLL_SN is set according to clause 5.3.3.2. It is initially set to 0.

The transmitting side of each AM RLC entity shall maintain the following counters:

a) PDU_WITHOUT_POLL – Counter

This counter is initially set to 0. It counts the number of AMD PDUs sent since the most recent poll bit was transmitted.

b) BYTE_WITHOUT_POLL – Counter

This counter is initially set to 0. It counts the number of data bytes sent since the most recent poll bit was transmitted.

c) RETX_COUNT – Counter

This counter counts the number of retransmissions of an RLC SDU or RLC SDU segment (see clause 5.3.2). There is one RETX_COUNT counter maintained per RLC SDU.

The receiving side of each AM RLC entity shall maintain the following state variables:

a) RX_Next – Receive state variable

This state variable holds the value of the SN following the last in-sequence completely received RLC SDU, and it serves as the lower edge of the receiving window. It is initially set to 0, and is updated whenever the AM RLC entity receives an RLC SDU with SN = RX_Next.

b) RX_Next_Status_Trigger – t-Reassembly state variable

This state variable holds the value of the SN following the SN of the RLC SDU which triggered t-Reassembly.

c) RX_Highest_Status – Maximum STATUS transmit state variable

This state variable holds the highest possible value of the SN which can be indicated by "ACK_SN" when a STATUS PDU needs to be constructed. It is initially set to 0.

d) RX_Next_Highest – Highest received state variable

This state variable holds the value of the SN following the SN of the RLC SDU with the highest SN among received RLC SDUs. It is initially set to 0.

[TS 38.331, clause 6.3.2]

t-Reassembly, t-ReassemblyExt

Timer for reassembly in TS 38.322 [4], in milliseconds. Value ms0 means 0 ms, value ms5 means 5 ms and so on. If t-ReassemblyExt-r17 is configured, the UE shall ignore t-Reassembly (without suffix).

7.1.2.3.9a.3	Test description

7.1.2.3.9a.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.2.1.1 with the exception that set to return no data in uplink and the AM DRB is configured according to Table 7.1.2.3.9a.3.1-1.

Table 7.1.2.3.9a.3.1-1: RLC parameters

| t-Reassembly | ms200 |
| --- | --- |
| t-ReassemblyExt-r17 | ms2200 |

System Simulator:

-	NR Cell 1 as specified in TS 38.508-1 [4] Table 4.4.2.1.

-	System information combination NR-29 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used.

UE:

-	The pre-configured UE location is defined in TS 38.508-1 [4] Clause 4.5C.

7.1.2.3.9a.3.2	Test procedure sequence

Table 7.1.2.3.9a.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits AMD PDU#1 containing the last segment (5 bytes) of RLC SDU#1 (SI field=10, SO=7). | <-- | AMD PDU#1 (SN=0) segment 2 | - | - |
| 2 | Check: For 2 sec after step 1, does the UE transmit an RLC STATUS PDU with ACK_SN=1, NACK_SN=0 with SOStart=0 and SOEnd=6? (Note 2) | --> | STATUS PDU (ACK_SN=1, NACK_SN=0 with SOStart=0 /SOEnd=6) | 1 | F |
| 3 | Wait for 2200 ms from step 1 to ensure that t-ReassemblyExt-r17 for the AMD PDU#1 expires at UE side. | - | - | - | - |
| 4 | Check: For 2 sec after step 3, does the UE transmit an RLC STATUS PDU with ACK_SN=1, NACK_SN=0 with SOStart=0 and SOEnd=6? (Note 3) | --> | STATUS PDU (ACK_SN=1, NACK_SN=0 with SOStart=0 /SOEnd=6) | 1 | P |
| Note 1: The RLC SDU size shall be 12 octets which are segmented into 7 and 5 octets. Note 2: The wait time of 2 sec is chosen to be greater than the propagation delays and transmissions of Scheduling Request and STATUS PDU, indicating that UE is not ignoring t-Reassembly. Note 3: The wait time of 2 sec is chosen to be greater than the propagation delays and transmissions of Scheduling Request and STATUS PDU, indicating that UE is ignoring t-Reassembly. |  |  |  |  |  |

7.1.2.3.9a.3.3	Specific message contents

Table 7.1.2.3.9a.3.3-1: RLC-Config (Preamble and all steps)

| Derivation Path: 38.508-1 [4], Table 4.6.3-149 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-Config ::= CHOICE { |  |  |  |
| am SEQUENCE { |  |  |  |
| dl-AM-RLC SEQUENCE { |  |  |  |
| t-Reassembly | ms200 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.2.3.9a.3.3-2: RLC-BearerConfig (Preamble and all steps)

| Derivation Path: 38.508-1 [4], Table 4.6.3-148 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-Config-v1700 ::= SEQUENCE { |  |  |  |
| dl-AM-RLC-v1700 SEQUENCE { |  |  |  |
| t-ReassemblyExt-r17 | ms2200 |  |  |
| } |  |  |  |
| } |  |  |  |

##### 7.1.2.3.10	AM RLC / Re-transmission of RLC PDU with and without re-segmentation

7.1.2.3.10.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives a STATUS PDU including a NACK_SN for missing AMD PDUs and missing AMD PDUs can be transmitted as indicated by lower layer at the particular transmission opportunity }

then { UE successfully retransmits missing AMD PDUs without re-segmentation }

}

(2)

with { UE in RRC_CONNECTED state }

ensure that {

when { NACK received for missing AMD PDUs and RETX_COUNT < maxRetxThreshold }

then { UE retransmits AMD PDUs }

}

(3)

with { UE in RRC_CONNECTED state }

ensure that {

when { AMD PDU to be retransmitted does not fit in new allocated TBS }

then { UE segments AMD PDU }

}

(4)

with { UE in RRC_CONNECTED state }

ensure that {

when { AMD PDU segment to be retransmitted does not fit in new allocated TBS }

then { UE re-segments AMD PDU segment to fit TBS }

}

7.1.2.3.10.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.322, clauses 5.3.2, 6.2.2.5, 6.2.3.3, 6.2.3.4, 6.2.3.5, 6.2.3.7, 6.2.3.10, 6.2.3.12, 6.2.3.14 and 6.2.3.15. Unless otherwise stated these are Rel-15 requirements.

[TS 38.322, clause 5.3.2]

The transmitting side of an AM RLC entity can receive a negative acknowledgement (notification of reception failure by its peer AM RLC entity) for an RLC SDU or an RLC SDU segment by the following:

-	STATUS PDU from its peer AM RLC entity.

When receiving a negative acknowledgement for an RLC SDU or an RLC SDU segment by a STATUS PDU from its peer AM RLC entity, the transmitting side of the AM RLC entity shall:

-	if the SN of the corresponding RLC SDU falls within the range TX_Next_Ack <= SN < = the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer:

-	consider the RLC SDU or the RLC SDU segment for which a negative acknowledgement was received for retransmission.

When an RLC SDU or an RLC SDU segment is considered for retransmission, the transmitting side of the AM RLC entity shall:

-	if the RLC SDU or RLC SDU segment is considered for retransmission for the first time:

-	set the RETX_COUNT associated with the RLC SDU to zero.

-	else, if it (the RLC SDU or the RLC SDU segment that is considered for retransmission) is not pending for retransmission already and the RETX_COUNT associated with the RLC SDU has not been incremented due to another negative acknowledgment in the same STATUS PDU:

-	increment the RETX_COUNT.

-	if RETX_COUNT = maxRetxThreshold:

-	indicate to upper layers that max retransmission has been reached.

When retransmitting an RLC SDU or an RLC SDU segment, the transmitting side of an AM RLC entity shall:

-	if needed, segment the RLC SDU or the RLC SDU segment;

-	form a new AMD PDU which will fit within the total size of AMD PDU(s) indicated by lower layer at the particular transmission opportunity;

-	submit the new AMD PDU to lower layer.

When forming a new AMD PDU, the transmitting side of an AM RLC entity shall:

-	only map the original RLC SDU or RLC SDU segment to the Data field of the new AMD PDU;

-	modify the header of the new AMD PDU in accordance with the description in sub clause 6.2.2.4;

-	set the P field according to sub clause 5.3.3.

[TS 38.322, clause 6.2.2.4]

AMD PDU consists of a Data field and an AMD PDU header. The AMD PDU header is byte aligned.

An AM RLC entity is configured by RRC to use either a 12 bit SN or a 18 bit SN. The length of the AMD PDU header is two and three bytes respectively.

An AMD PDU header contains a D/C, a P, a SI, and a SN. An AMD PDU header contains the SO field only when the Data field consists of an RLC SDU segment which is not the first segment, in which case a 16 bit SO is present.

Figure 6.2.2.4-1: AMD PDU with 12 bit SN (No SO)

Figure 6.2.2.4-2: AMD PDU with 18 bit SN (No SO)

Figure 6.2.2.4-3: AMD PDU with 12 bit SN with SO

Figure 6.2.2.4-4: AMD PDU with 18 bit SN with SO

[TS 38.322, clause 6.2.2.5]

STATUS PDU consists of a STATUS PDU payload and an RLC control PDU header.

RLC control PDU header consists of a D/C and a CPT field.

The STATUS PDU payload starts from the first bit following the RLC control PDU header, and it consists of one ACK_SN and one E1, zero or more sets of a NACK_SN, an E1, an E2 and an E3, and possibly a pair of a SOstart and a SOend or a NACK range field for each NACK_SN.

Figure 6.2.2.5-1: STATUS PDU with 12 bit SN

Figure 6.2.2.5-2: STATUS PDU with 18 bit SN

[TS 38.322, clause 6.2.3.3]

Length: 12 bits or 18 bits (configurable) for AMD PDU. 6 bits or 12 bits (configurable) for UMD PDU.

The SN field indicates the sequence number of the corresponding RLC SDU. For RLC AM, the sequence number is incremented by one for every RLC SDU. For RLC UM, the sequence number is incremented by one for every segmented RLC SDU.

[TS 38.322, clause 6.2.3.4]

Length: 2 bits.

The SI field indicates whether an RLC PDU contains a complete RLC SDU or the first, middle, last segment of an RLC SDU.

Table 6.2.3.4-1: SI field interpretation

| Value | Description |
| --- | --- |
| 00 | Data field contains all bytes of an RLC SDU |
| 01 | Data field contains the first segment of an RLC SDU |
| 10 | Data field contains the last segment of an RLC SDU |
| 11 | Data field contains neither the first nor last segment of an RLC SDU |

[TS 38.322, clause 6.2.3.5]

Length: 16 bits

The SO field indicates the position of the RLC SDU segment in bytes within the original RLC SDU. Specifically, the SO field indicates the position within the original RLC SDU to which the first byte of the RLC SDU segment in the Data field corresponds. The first byte of the original RLC SDU is referred by the SO field value "0000000000000000", i.e., numbering starts at zero.

[TS 38.322, clause 6.2.3.7]

Length: 1 bit.

The P field indicates whether or not the transmitting side of an AM RLC entity requests a STATUS report from its peer AM RLC entity. The interpretation of the P field is provided in Table 6.2.3.7-1.

Table 6.2.3.7-1: P field interpretation

| Value | Description |
| --- | --- |
| 0 | Status report not requested |
| 1 | Status report is requested |

[TS 38.322, clause 6.2.3.10]

Length: 12 bits or 18 bits (configurable).

The ACK_SN field indicates the SN of the next not received RLC SDU which is not reported as missing in the STATUS PDU. When the transmitting side of an AM RLC entity receives a STATUS PDU, it interprets that all RLC SDUs up to but not including the RLC SDU with SN = ACK_SN have been received by its peer AM RLC entity, excluding those RLC SDUs indicated in the STATUS PDU with NACK_SN, portions of RLC SDUs indicated in the STATUS PDU with NACK_SN, SOstart and SOend, RLC SDUs indicated in the STATUS PDU with NACK_SN and NACK_range, and portions of RLC SDUs indicated in the STATUS PDU with NACK_SN, NACK range, SOstart and SOend.

[TS 38.322, clause 6.2.3.12]

Length: 12 bits or 18 bits (configurable).

The NACK_SN field indicates the SN of the RLC SDU (or RLC SDU segment) that has been detected as lost at the receiving side of the AM RLC entity.

[TS 38.322, clause 6.2.3.14]

Length: 16 bits.

The SOstart field (together with the SOend field) indicates the portion of the RLC SDU with SN = NACK_SN (the NACK_SN for which the SOstart is related to) that has been detected as lost at the receiving side of the AM RLC entity. Specifically, the SOstart field indicates the position of the first byte of the portion of the RLC SDU in bytes within the original RLC SDU. The first byte of the original RLC SDU is referred by the SOstart field value "0000000000000000", i.e., numbering starts at zero.

[TS 38.322, clause 6.2.3.15]

Length: 16 bits.

When E3 is 0, the SOend field (together with the SOstart field) indicates the portion of the RLC SDU with SN = NACK_SN (the NACK_SN for which the SOend is related to) that has been detected as lost at the receiving side of the AM RLC entity. Specifically, the SOend field indicates the position of the last byte of the portion of the RLC SDU in bytes within the original RLC SDU. The first byte of the original RLC SDU is referred by the SOend field value "0000000000000000", i.e., numbering starts at zero. The special SOend value "1111111111111111" is used to indicate that the missing portion of the RLC SDU includes all bytes to the last byte of the RLC SDU.

When E3 is 1, the SOend field indicates the portion of the RLC SDU with SN = NACK_SN + NACK range - 1 that has been detected as lost at the receiving side of the AM RLC entity. Specifically, the SOend field indicates the position of the last byte of the portion of the RLC SDU in bytes within the original RLC SDU. The first byte of the original RLC SDU is referred by the SOend field value "0000000000000000", i.e., numbering starts at zero. The special SOend value "1111111111111111" is used to indicate that the missing portion of the RLC SDU includes all bytes to the last byte of the RLC SDU.

7.1.2.3.10.3	Test description

7.1.2.3.10.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.2.1.1 with the exception for the AM DRB is configured according to Tables 7.1.2.3.10.3.1-1.

Table 7.1.2.3.10.3.1-1: RLC settings

| Parameter | Value |
| --- | --- |
| t-PollRetransmit | ms150 |

7.1.2.3.10.3.2	Test procedure sequence

Table 7.1.2.3.10.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits one AMD PDU containing SDU#1 (91 bytes) in its data field. | <-- | AMD PDU#1 | - | - |
| 2 | The UE transmits one AMD PDU containing SDU#1 in its data field. | --> | AMD PDU#1 (SN=0) | - | - |
| 3 | The SS transmits one AMD PDU containing SDU#2 (91 bytes) in its data field. | <-- | AMD PDU#2 | - | - |
| 4 | The UE transmits one AMD PDU containing SDU#2 in its data field. | --> | AMD PDU#2 (SN=1) | - | - |
| 5 | The SS transmits a RLC STATUS PDU. ACK_SN=2, NACK_SN=0. | <-- | STATUS PDU | - | - |
| 6 | Check: Does the UE transmit the AMD PDU not yet acknowledged? | --> | AMD PDU#1 (SN=0) | 1 | P |
| 7 | The SS transmits a RLC STATUS PDU. ACK_SN=2. | <-- | STATUS PDU | - | - |
| 8 | The SS transmits one AMD PDU containing SDU#3 (91 bytes) in its data field. | <-- | AMD PDU#3 | - | - |
| 9 | The UE transmits an AMD PDU containing SDU#3 in its data field. | --> | AMD PDU#3 (SN=2) | - | - |
| - | EXCEPTION: Steps 10 to 11 shall be repeated until RETX_COUNT= maxRetxThreshold-1. | - | - | - | - |
| 10 | The SS transmits a RLC STATUS PDU. ACK_SN=3 and NACK_SN=2. | <-- | STATUS PDU | - | - |
| 11 | Check: Does the UE retransmit the AMD PDU not yet acknowledged? | --> | AMD PDU#3 (SN=2) | 2 | P |
| 12 | The SS transmits a RLC STATUS PDU. ACK_SN=3. | <-- | STATUS PDU | - | - |
| 13 | The SS stops the UL grant transmission. | - | - | - | - |
| 14 | The SS transmits one AMD PDU containing SDU#4 (91 bytes) in its data field. | <-- | AMD PDU#4 (SN=3) | - | - |
| 15 | The SS waits for 60 ms and allocates one UL grant of size 808 bits. (Note 1) | <-- | (UL grant, 808 bits) | - | - |
| 16 | The UE transmits an AMD PDU with the same data contents as received in the corresponding part of SDU#4? | --> | AMD PDU#4 (SN=3) | - | - |
| 17 | The SS transmits a STATUS PDU. This PDU nacks the AMD PDU with SN=3. ACK_SN=4 and NACK_SN=3. | <-- | STATUS PDU | - | - |
| 18 | The SS waits for 100 ms and then allocates 1 UL grant of size 408 bits (Note 2) | <-- | (UL grant, 408 bits) | - | - |
| 18A | The SS waits for 20 ms and then allocates 1 UL grant of size 456 bits (Note 5) | <-- | (UL grant, 456 bits) | - | - |
| 19 | Check: Does the UE transmit an SDU segment with SI=01 and SOEnd=43+X (Note 7) and the same data contents at the received positions as in the original SDU#4? | --> | SDU#4 segment 1 (SN=3) | 3 | P |
| 20 | Check: Does the UE transmit an SDU segment with SI=10 and SOStart=44+X (Note 7) and the same data contents at the received positions as in the original SDU#4? | --> | SDU#4 segment 2 (SN=3) | 3 | P |
| 21 | After 100 ms SS transmits a STATUS PDU. This PDU nacks the SDU with SN=3. NACK_SN=3, SOStart=0, SOEnd=43+X (Note 7) and ACK_SN=4. | <-- | STATUS PDU | - | - |
| 22 | The SS waits for 100 ms and then allocates 2 UL grants (UL grant allocation type 2) at an interval of 20 ms of size 240 bits (Note 3, Note 6) | <-- | (UL grant, 240 bits) | - | - |
| 22A | Void |  |  | - | - |
| 23 | Check: Does the UE transmit an AMD PDU segment with SI=01 and SOEnd=22+X (Note 7) and the same data contents at the received positions as in the original SDU#4? | --> | SDU#4 segment 1, first part (SN=3) | 4 | P |
| 24 | Check: Does the UE transmit an AMD PDU segment with SI=11, SOStart=23+X (Note 7) and the same data contents at the received positions as in the original SDU#4? | --> | SDU#4 segment 1, second part (SN=3) | 4 | P |
| 25 | The SS transmits a STATUS PDU. This PDU acks the AMD PDUs with SN=3. ACK_SN=4. | <-- | STATUS PDU | - | - |
| Note 1: UL grant of 808 bits=101 bytes (LRBs & IMCS as per 38.523-3[3] annex B) is chosen to allow the UE to transmit one PDU (91 bytes RLC SDU and 3 bytes RLC Header and 2 bytes MAC Sub PDU header and 2 bytes short BSR + 3 bytes padding or 5 bytes padding) for (pc_supportOfRedCap_r17=false AND pc_supportOfERedCap_r18= false), or one PDU (91 bytes RLC SDU and 2 bytes RLC Header and 2 bytes MAC Sub PDU header and 2 bytes short BSR + 4 bytes padding or 6 bytes padding) for (pc_supportOfRedCap_r17=true OR pc_supportOfERedCap_r18=true) at a time. Note 2: UL grant of 408 bits (LRBs & IMCS as per 38.523-3[3] annex B) is chosen such that UE will segment into 2 AMD PDUs. MAC PDU of 408 bits=51 bytes fit an AMD PDU payload of 44 bytes + 3 bytes for the segment 1 of the AMD PDU header + 2 bytes for MAC header + 2 bytes of MAC BSR CE for (pc_supportOfRedCap_r17 =false AND pc_supportOfERedCap_r18= false), or an AMD PDU payload of 45 bytes + 2 bytes for the segment 1 of the AMD PDU header + 2 bytes for MAC header + 2 bytes of MAC BSR CE for (pc_supportOfRedCap_r17=true OR pc_supportOfERedCap_r18=true). Note 3: UL grant of 240 bits (LRBs & IMCS as per 38.523-3[3] annex B) is chosen such that UE will segment into 2 AMD PDUs. MAC PDU of 240 bits=30 bytes fit an AMD PDU payload of = 23 bytes + 3 bytes for the segment 1 first part of the AMD PDU header + 2 bytes for MAC header + 2 bytes for MAC BSR CE for (pc_supportOfRedCap_r17=false AND pc_supportOfERedCap_r18= false ), or an AMD PDU payload of = 24 bytes + 2 bytes for the segment 1 first part of the AMD PDU header + 2 bytes for MAC header + 2 bytes for MAC BSR CE for (pc_supportOfRedCap_r17=true OR pc_supportOfERedCap_r18=true). Note 4: Void. Note 5: UL grant of 456 bits (LRBs & IMCS as per 38.523-3[3] annex B) is chosen such that UE will segment into 2 AMD PDUs. MAC PDU of 456 bits=57 bytes fit an AMD PDU payload of 47 bytes + 5 bytes for the segment 2 of the AMD PDU header + 2 bytes for MAC header +2 bytes for possible short BSR and 1 byte padding or only 3 bytes padding for (pc_supportOfRedCap_r17=false AND pc_supportOfERedCap_r18= false), or an AMD PDU payload of 46 bytes + 4 bytes for the segment 2 of the AMD PDU header + 2 bytes for MAC header +2 bytes for possible short BSR and 3 byte padding or only 5 bytes padding for (pc_supportOfRedCap_r17=true OR pc_supportOfERedCap_r18=true). Note 6: UL grant of 240 bits (LRBs & IMCS as per 38.523-3[3] annex B) is chosen such that UE will segment into 2 AMD PDUs. MAC PDU of 240 bits=30 bytes fit an AMD PDU payload of 21 bytes + 5 bytes for the segment 1 second part of the AMD PDU header + 2 bytes for MAC header +2 bytes for possible short BSR or padding for (pc_supportOfRedCap_r17=false AND pc_supportOfERedCap_r18= false), or an AMD PDU payload of 21 bytes + 4 bytes for the segment 1 second part of the AMD PDU header + 2 bytes for MAC header +2 bytes for possible short BSR and 1 bytes padding or only 3 bytes padding for (pc_supportOfRedCap_r17=true OR pc_supportOfERedCap_r18=true). Note 7: X=0 for (pc_supportOfRedCap_r17=false AND pc_supportOfERedCap_r18= false) and X=1 for (pc_supportOfRedCap_r17=true OR pc_supportOfERedCap_r18=true). |  |  |  |  |  |

7.1.2.3.10.3.3	Specific message contents

None.

##### 7.1.2.3.11	AM RLC / RLC re-establishment procedure

7.1.2.3.11.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { RLC re-establishment is performed upon request by RRC }

then { The UE discards all RLC SDUs, RLC SDU segments, and RLC PDUs, if any }

}

(2)

with { UE in RRC_CONNECTED state }

ensure that {

when { RLC re-establishment is performed upon request by RRC }

then { The UE resets all state variables to their initial values }

}

7.1.2.3.11.2	Conformance requirements

References:

The conformance requirements covered in the present test case are specified in: TS 38.322, clauses 5.1.2, 7.1 and TS 38.331 clause 5.3.11. Unless otherwise stated these are Rel-15 requirements.

[TS 38.322, clause 5.1.2]

When upper layers request an RLC entity re-establishment, the UE shall:

-	discard all RLC SDUs, RLC SDU segments, and RLC PDUs, if any;

-	stop and reset all timers;

-	reset all state variables to their initial values.

[TS 38.322, clause 7.1]

This sub clause describes the state variables used in AM and UM entities in order to specify the RLC protocol. The state variables defined in this subclause are normative.

All state variables and all counters are non-negative integers.

All state variables related to AM data transfer can take values from 0 to 4095 for 12 bit SN or from 0 to 262143 for 18 bit SN. All arithmetic operations contained in the present document on state variables related to AM data transfer are affected by the AM modulus (i.e. final value = [value from arithmetic operation] modulo 4096 for 12 bit SN and 262144 for 18 bit SN).

All state variables related to UM data transfer can take values from 0 to 63 for 6 bit SN or from 0 to 4095 for 12 bit SN. All arithmetic operations contained in the present document on state variables related to UM data transfer are affected by the UM modulus (i.e. final value = [value from arithmetic operation] modulo 64 for 6 bit SN and 4096 for 12 bit SN).

When performing arithmetic comparisons of state variables or SN values, a modulus base shall be used.

TX_Next_Ack and RX_Next shall be assumed as the modulus base at the transmitting side and receiving side of an AM RLC entity, respectively. This modulus base is subtracted from all the values involved, and then an absolute comparison is performed (e.g. RX_Next <= SN < RX_Next + AM_Window_Size is evaluated as [RX_Next – RX_Next] modulo 2[sn-FieldLength] <= [SN – RX_Next] modulo 2[sn-FieldLength] < [RX_Next + AM_Window_Size – RX_Next] modulo 2[sn-FieldLength]), where sn-FieldLength is 12 or 18 for 12 bit SN and 18 bit SN, respectively.

RX_Next_Highest– UM_Window_Size shall be assumed as the modulus base at the receiving side of an UM RLC entity. This modulus base is subtracted from all the values involved, and then an absolute comparison is performed (e.g. (RX_Next_Highest– UM_Window_Size) <= SN < RX_Next_Highest is evaluated as [(RX_Next_Highest– UM_Window_Size) – (RX_Next_Highest– UM_Window_Size)] modulo 2[sn-FieldLength] <= [SN – (RX_Next_Highest– UM_Window_Size)] modulo 2[sn-FieldLength] < [RX_Next_Highest– (RX_Next_Highest– UM_Window_Size)] modulo 2[sn-FieldLength]), where sn-FieldLength is 6 or 12 for 6 bit SN and 12 bit SN, respectively.

The transmitting side of each AM RLC entity shall maintain the following state variables:

a) TX_Next_Ack – Acknowledgement state variable

This state variable holds the value of the SN of the next RLC SDU for which a positive acknowledgment is to be received in-sequence, and it serves as the lower edge of the transmitting window. It is initially set to 0, and is updated whenever the AM RLC entity receives a positive acknowledgment for an RLC SDU with SN = TX_Next_Ack.

b) TX_Next – Send state variable

This state variable holds the value of the SN to be assigned for the next newly generated AMD PDU. It is initially set to 0, and is updated whenever the AM RLC entity constructs an AMD PDU with SN = TX_Next and contains an RLC SDU or the last segment of a RLC SDU.

c) POLL_SN – Poll send state variable

This state variable holds the value of the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer when POLL_SN is set according to sub clause 5.3.3.2. It is initially set to 0.

The transmitting side of each AM RLC entity shall maintain the following counters:

a) PDU_WITHOUT_POLL – Counter

This counter is initially set to 0. It counts the number of AMD PDUs sent since the most recent poll bit was transmitted.

b) BYTE_WITHOUT_POLL – Counter

This counter is initially set to 0. It counts the number of data bytes sent since the most recent poll bit was transmitted.

c) RETX_COUNT – Counter

This counter counts the number of retransmissions of an RLC SDU or RLC SDU segment (see subclause 5.3.2). There is one RETX_COUNT counter maintained per RLC SDU.

The receiving side of each AM RLC entity shall maintain the following state variables:

a) RX_Next – Receive state variable

This state variable holds the value of the SN following the last in-sequence completely received RLC SDU, and it serves as the lower edge of the receiving window. It is initially set to 0, and is updated whenever the AM RLC entity receives an RLC SDU with SN = RX_Next.

b) RX_Next_Status_Trigger – t-Reassembly state variable

This state variable holds the value of the SN following the SN of the RLC SDU which triggered t-Reassembly.

c) RX_Highest_Status – Maximum STATUS transmit state variable

This state variable holds the highest possible value of the SN which can be indicated by "ACK_SN" when a STATUS PDU needs to be constructed. It is initially set to 0.

d) RX_Next_Highest – Highest received state variable

This state variable holds the value of the SN following the SN of the RLC SDU with the highest SN among received RLC SDUs. It is initially set to 0.

Each transmitting UM RLC entity shall maintain the following state variables:

a) TX_Next

This state variable holds the value of the SN to be assigned for the next newly generated UMD PDU with segment. It is initially set to 0, and is updated after the UM RLC entity submits a UMD PDU including the last segment of an RLC SDU to lower layers.

Each receiving UM RLC entity shall maintain the following state variables and constant:

b) RX_Next_Reassembly – UM receive state variable

This state variable holds the value of the earliest SN that is still considered for reassembly. It is initially set to 0.

c) RX_Timer_Trigger – UM t-Reassembly state variable

This state variable holds the value of the SN following the SN which triggered t-Reassembly.

d) RX_Next_Highest– UM receive state variable

This state variable holds the value of the SN following the SN of the UMD PDU with the highest SN among received UMD PDUs. It serves as the higher edge of the reassembly window. It is initially set to 0.

[TS 38.331, clause 5.3.11]

UE shall:

1>	reset MAC;

1>	if T302 is running:

2>	stop timer T302;

2>	perform the actions as specified in 5.3.14.4;

1>	stop all timers that are running except T320 and T325;

1>	discard the UE Inactive AS context;

1>	set the variable pendingRnaUpdate to false, if that is set to true;

1>	discard the KgNB, the KRRCenc key, the KRRCint, the KUPint key and the KUPenc key, if any;

1>	release all radio resources, including release of the RLC entity, the MAC configuration and the associated PDCP entity and SDAP for all established RBs;

1>	indicate the release of the RRC connection to upper layers together with the release cause;

1>	enter RRC_IDLE and perform cell selection as specified in TS 38.304 [20], except if going to RRC_IDLE was triggered by selecting an inter-RAT cell while T311 was running;

1>	if going to RRC_IDLE was triggered by reception of the RRCRelease message including a waitTime:

2>	start timer T302 with the value set to the waitTime;

2>	inform the upper layer that access barring is applicable for all access categories except categories '0' and '2'.

7.1.2.3.11.3	Test description

7.1.2.3.11.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.2.1.1 with the exception for the AM DRB is configured according to Table 7.1.2.3.11.3.1-1.

Table 7.1.2.3.11.3.1-1: RLC parameters

| Parameter | Value |
| --- | --- |
| t-Reassembly | ms200 |
| t-PollRetransmit | ms4000 |

Table 7.1.2.3.11.3.1-2: PDCP parameters

| Parameter | Value |
| --- | --- |
| t-Reordering | ms160 |
| statusReportRequired | Not present |

7.1.2.3.11.3.2	Test procedure sequence

Table 7.1.2.3.11.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | The SS ignores scheduling requests and does not allocate any uplink grant. | - | - | - | - |
| 1 | The SS creates 2 RLC SDUs of size 40 bytes segmented into two AMD PDUs each. AMD PDU#1 and AMD PDU#2 belong to RLC SDU#1, AMD PDU#3 and #4 belong to RLC SDU#2. SS transmits AMD PDU#1 (SN=0), AMD PDU#2 (SN=0) and AMD PDU#4 (SN=1). | <-- | AMD PDU#1 AMD PDU#2 AMD PDU#4 | - | - |
| 2 | 60 ms after sending PDU#1 in step 1 the SS allocates 1 UL grant of default size. | <-- | (UL grant) | - | - |
| 3 | The UE returns RLC SDU#1. | --> | (RLC SDU#1) | - | - |
| 4 | The SS does not acknowledge the reception of RLC SDU#1. | - | - | - | - |
| 5 | The SS transmits NR RRCReconfiguration message to trigger RLC re-establishment on DRB using Reconfig with sync procedure. (Note 1) (Note 4) (Note 6) (Note 7) | <-- | RRCReconfiguration | - | - |
| 6 | The SS starts the UL default grant transmissions and for NR sets UL grant in Random Access Msg4 to allow the UE to send step7 and step 8 together. (Note 10) | - | - | - | - |
| - | EXCEPTION: Steps 7 and 8 can occur in any order. (Note 8) | - | - | - | - |
| 7 | The UE transmits a NR RRCReconfigurationComplete message. (Note 5) | --> | RRCReconfigurationComplete | - | - |
| 8 | The UE retransmits RLC SDU #1 (SN=0). (Note 2) | --> | (RLC SDU#1) | - | - |
| 9 | SS transmits a STATUS PDU (ACK_SN = 1). | <-- | STATUS PDU | - | - |
| 10 | SS transmits AMD PDU#3 with SN=0 and the P field set to "1" | <-- | AMD PDU#3 | - | - |
| 11 | Void |  |  |  |  |
| 12 | Check: For 1 second, does the UE return RLC SDU#2 (Note 3) | --> | (RLC SDU#2) | 1 | F |
| 13 | SS transmits AMD PDU#4 with SN=0 | <-- | AMD PDU#4 | - | - |
| 13A | UE transmits a STATUS PDU (ACK_SN=1) (Note 9) | --> | STATUS PDU | - | - |
| 14 | Check: Does the UE return RLC SDU#2 with SN=1? (Note 9) | --> | (RLC SDU#2) | 2 | P |
| 15 | SS transmits a STATUS PDU (ACK_SN = 2) | <-- | STATUS PDU | - | - |
| Note 1: Upon a RLC re-establishment a conformant UE discards any remaining AMD PDUs in the receiver and transmitter side, stops and resets all timers and resets all state variables to their initial values. Note 2: The UE will retransmit the PDCP SDU associated with RLC SDU#1 in accordance to TS 38.323 clause 5.5 Note 3: AMD PDU#4 is discarded by a conformant UE in step 5. Note 4: For EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration 36.508 [7], Table 4.6.1-8 using condition EN-DC_EmbedNR_RRCRecon, EN-DC_PSCell_HO and RBConfig_NoKeyChange. Note 5: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. Note 6: For NR, the RRCReconfiguration message is as per RRCReconfiguration-HO with condition RBConfig_NoKeyChange according to 38.508-1 [4], Table 4.8.1-1A. Note 7: The expiry of t-Reassembly timer started in step 1 will trigger STATUS PDU from the UE. This starts SR. The RRC reconfiguration is scheduled 300 ms in advance. sr-transMax is therefore set to n64 so that the UE does not reach sr-transMax and then RACHes before reconfiguring. Note 8: Per 38.508-1 Table 4.6.3-66: LogicalChannelConfig, both SRB1 and DRB have the same logical channel priority with prioritisedBitRate as infinity. Note 9 RLC PDUs at steps 13A and 14 may be received by the SS in the same slot or in multiple slots (max one MAC PDU in a slot). Note 10: For NR, UL grant in Random Access Msg4 of 480 bits (LRBs & IMCS as per 38.523-3[3] annex B) is chosen such that UE is allowed to send 12 bytes for RRCReconfigurationComplete + 44 or 45 bytes for RLC SDU #1 + 3 or 4 bytes for MAC BSR or padding. The MAC subPDU size for RRCReconfigurationComplete is 12 bytes with 2 bytes MAC Header+ 2 bytes RLC Header+6 bytes PDCP Header with MAC-I+2bytes RRC message. The MAC subPDU size for RLC SDU #1 is 44 bytes with 2 bytes MAC Header + 2 bytes RLC Header+40 bytesRLC SDU for (pc_supportOfRedCap_r17=true OR pc_supportOfERedCap_r18=true), or 45 bytes with 2 bytes MAC Header + 3 bytes RLC Header+40 bytesRLC SDU for (pc_supportOfRedCap_r17=false AND pc_supportOfERedCap_r18= false). |  |  |  |  |  |

7.1.2.3.11.3.3	Specific message contents

Table 7.1.2.3.11.3.3-0: SchedulingRequest-Config (Preamble)

| Derivation Path: 38.508-1 [4], Table 4.6.3-155 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| sr-TransMax | n64 | 7.1.2.3.11.3.2-1 Note 7 |  |

Table 7.1.2.3.11.3.3-1: RRCReconfiguration-HO (step 5, Table 7.1.2.3.11.3.3-1)

| Derivation Path: 38.508-1 [4], Table 4.8.1-1A with condition RBConfig_NoKeyChange |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig | According to Table 7.1.2.3.11.3.3-1A |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig | According to Table 7.1.2.3.11.3.3-1B |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.2.3.11.3.3-2: Void

Table 7.1.2.3.11.3.3-1A: RadioBearerConfig

| Derivation Path: 38.508-1 [4], Table 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | n entries | n is equal to the total number of DRBs established during preamble |  |
| DRB-ToAddMod[k=1..n] SEQUENCE { |  | entry (1..n) |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config | SDAP-Config | According to TS 38.508-1, Table 4.6.3-161 |  |
| } |  |  |  |
| drb-Identity | k | k=1..n |  |
| reestablishPDCP | Not present |  |  |
| recoverPDCP | true |  |  |
| pdcp-Config | PDCP-Config | According to TS 38.508-1, Table 4.6.3-99 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.2.3.11.3.3-1B: CellGroupConfig

| Derivation Path: 38.508-1 [4], Table 4.6.1-13 with condition PCell_change |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 2+n entries | n is equal to the total number of DRBs established during preamble |  |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions SRB1 and Re-establish_RLC | entry 1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions SRB2 and Re-establish_RLC | entry 2 |  |
| RLC-BearerConfig[k+2, k=1..n] | RLC-BearerConfig with conditions AM, DRBk and Re-establish_RLC | entry [k+2, k=1..n] |  |
| } |  |  |  |

##### 7.1.2.3.12	AM RLC / LTM cell switch / SCG / Secondary Key

7.1.2.3.12.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with candidate configuration for LTM SCG and the SRB/DRB are using the secondary key }

ensure that {

when { UE receives Random Access Resources in LTM Cell Switch Command MAC CE and LTM-Candidate IE in ltm-Config does not contain the field ltm-NoResetID and the UE does not have any value stored of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID }

then { UE re-establish the RLC entity for each logicalChannelIdentity and logicalChannelIdentityExt that is part of the current UE configuration }

}

(2)

with { UE in RRC_Connected state with candidate configuration for LTM SCG and the SRB/DRB are using the secondary key }

ensure that {

when { UE reselects to an LTM cell and the value of field ltm-NoResetID contained within the LTM-Candidate IE in ltm-Config for the selected cell is not equal to the value of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID }

then { UE re-establish the RLC entity for each logicalChannelIdentity and logicalChannelIdentityExt that is part of the current UE configuration }

}

(3)

with { UE in RRC_Connected state with candidate configuration for LTM SCG and the SRB/DRB are using the secondary key }

ensure that {

when { UE reselects to an LTM cell and the value of field ltm-NoResetID contained within the LTM-Candidate IE in ltm-Config for the selected cell is equal to the value of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID }

then { UE does not re-establish the RLC entity for any logicalChannelIdentity and logicalChannelIdentityExt that is part of the current UE configuration }

}

7.1.2.3.12.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331 clauses 5.3.5.18.6 and 5.3.1.2, TS 38.322 clauses 5.1.2 and 7.1. Unless otherwise stated these are Rel-18 requirements.

[TS 38.331, clause 5.3.5.18.6]

Upon the indication by lower layers that an LTM cell switch procedure is triggered, or upon performing LTM cell switch following cell selection performed while timer T311 was running, as specified in 5.3.7.3, the UE shall:

1>	if the LTM cell switch is triggered on the MCG:

2>	release/clear all current dedicated and common radio configurations which have neither been received via SRB1 within mrdc-SecondaryCellGroup, nor via SRB3 except for the following:

-	the radio bearer configuration (configured via RadioBearerConfig)

-	the logicalChannelIdentity and logicalChannelIdentityExt of RLC bearers configured in RLC-BearerConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the bh-LogicalChannelIdentity of BH RLC channels configured in BH-RLC-ChannelConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the UE variables VarLTM-ServingCellNoResetID and VarLTM-ServingCellUE-MeasuredTA-ID;

-	the ltm-Config;

-	the MCG C-RNTI;

-	the AS security configurations associated with the master key;

-	the logged measurement configuration;

1>	else, if the LTM cell switch is triggered on the SCG:

2>	release/clear all current dedicated and common radio configurations which have been received either via SRB1 within mrdc-SecondaryCellGroup, or via SRB3 except for the following:

-	the radio bearer configuration (configured via RadioBearerConfig IE)

-	the logicalChannelIdentity and logicalChannelIdentityExt of RLC bearers configured in RLC-BearerConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the bh-LogicalChannelIdentity of BH RLC channels configured in BH-RLC-ChannelConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the UE variables VarLTM-ServingCellNoResetID and VarLTM-ServingCellUE-MeasuredTA-ID;

-	the ltm-Config;

-	the AS security configurations associated with the secondary key;

1>	for each SRB/DRB in the current UE configuration:

2>	if the LTM cell switch is triggered on the MCG and the SRB/DRB using the master key; or

2>	if the LTM cell switch is triggered on the SCG and the SRB/DRB using the secondary key:

3>	keep the associated PDCP and SDAP entities, their state variables, buffers and timers;

3>	release all fields related to the SRB/DRB configuration except for srb-Identity and drb-Identity;

3>	apply the default SRB configuration defined in 9.2.1 for the corresponding SRB;

NOTE 00:	For all radio bearers and RLC bearers included in the LTM candidate configuration to be applied at an LTM cell switch execution (i.e., as derived from the LTM reference configuration and the LTM candidate configuration), even if those radio bearers and RLC bearers were configured before the LTM cell switch execution, the network includes fields as specified for the initial setup of radio bearers and RLC bearers and sets the values previously stored by the UE for the fields that cannot be modified according to presence conditions or field descriptions.

1>	apply the default L1 parameter values as specified in corresponding physical layer specifications except for the parameters for which values are provided in SIB1;

1>	use the default values specified in 9.2.3 for timers T310, T311 and constants N310, N311 associated with the cell group for which the LTM cell switch procedure is triggered, where T310, N310, and N311 are for both MCG and SCG, and T311 is only for the MCG;

1>	apply the default MAC Cell Group configuration as specified in 9.2.2 for the cell group for which the LTM cell switch procedure is triggered;

1>	if the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 does not contain the field ltm-NoResetID and if the UE does not have any value stored of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID; or

1>	if the value of field ltm-NoResetID contained within the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID:

2>	for each logicalChannelIdentity and logicalChannelIdentityExt that is part of the current UE configuration for the cell group for which the LTM cell switch procedure is triggered:

3>	if servedRadioBearer is set to drb-Identity:

4>	after the end of this procedure, re-establish the corresponding RLC entity as specified in TS 38.322 [4], after applying the LTM configuration in ltm-CandidateConfig within the LTM-Candidate IE in ltm-Config;

2>	for each bh-LogicalChannelIdentity that is part of the current UE configuration for the cell group for which the LTM cell switch procedure is triggered:

3>	after the end of this procedure, re-establish the corresponding RLC entity as specified in TS 38.322 [4], after applying the LTM configuration in ltm-CandidateConfig within the LTM-Candidate IE in ltm-Config;

2>	for each drb-Identity value that is part of the current UE configuration:

3>	if this DRB is an AM DRB:

4>	after the end of this procedure, trigger the PDCP entity of this DRB to perform data recovery as specified in TS 38.323 [5], after applying the LTM configuration in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config;

2>	if the value of field ltm-NoResetID contained within the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID:

3>	replace the value of ltm-ServingCellNoResetID in VarLTM-ServingCellNoResetID with the value of ltm-NoResetID in the LTM-Candidate in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3;

1> if the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 contains the field ltm-UE-MeasuredTA-ID:

2>	if the value of ltm-UE-MeasuredTA-ID is not equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID:

3>	replace the value of ltm-ServingCellUE-MeasuredTA-ID in VarLTM-ServingCellUE-MeasuredTA-ID with the value received within ltm-UE-MeasuredTA-ID;

3>	for each LTM-Candidate IE in ltm-Config:

4>	if the value of ltm-UE-MeasuredTA-ID within LTM-Candidate IE is equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID:

5>	inform lower layers that the UE is configured with UE-based TA measurements for the LTM-Candidate;

4>	else:

5>	inform lower layers that the UE is not configured with UE-based TA measurements for the LTM-Candidate;

NOTE 0:	The UE is not expected to perform UE-based TA measurements for an SpCell.

1>	if ltm-ConfigComplete is not included within the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3:

2>	consider ltm-ReferenceConfiguration in ltm-Config, associated with the cell group for which the LTM cell switch procedure is triggered, to be the current UE configuration for the fields and configurations to be released by the actions above in this procedure;

2>	if measConfig is included within ltm-ReferenceConfiguration in ltm-Config;

3>	perform the measurement configuration procedure as specified in clause 5.5.2 by considering the measConfig within ltm-ReferenceConfiguration in ltm-Config as the received measConfig:

NOTE 1:	When the UE considers the reference configuration to be the current UE configuration, the UE should store fields and configurations that are part of the reference configuration but should not execute any actions or procedures triggered by the reception of an RRCReconfiguration message which are described in clause 5.3.5.3, unless specified otherwise in this clause.

1>	if the LTM cell switch is triggered by an indication from lower layers:

2>	apply the RRCReconfiguration message in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config identified by the LTM candidate configuration identity received from lower layers according to clause 5.3.5.3;

1>	else (LTM cell switch triggered upon cell selection performed while timer T311 was running):

2>	apply the RRCReconfiguration message in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config related to the LTM candidate configuration identity for the selected cell (i.e., in accordance with 5.3.7.3) according to clause 5.3.5.3;

1>	if the LTM cell switch is triggered on the MCG:

2>	release the radio bearer(s) using the master key and the MCG logical channel(s) that were part of the UE configuration before this LTM cell switch procedure but not part of the LTM candidate configuration either indicated by lower layers or for the selected cell in accordance with 5.3.7.3, or the LTM reference configuration (in case the LTM candidate configuration does not include ltm-ConfigComplete);

1>	else, if the LTM cell switch is triggered on the SCG:

2>	release the radio bearer(s) using the secondary key and the SCG logical channel(s) that were part of the UE configuration before this LTM cell switch procedure but not part of the LTM candidate configuration either indicated by lower layers or for the selected cell in accordance with 5.3.7.3, or the LTM reference configuration (in case the LTM candidate configuration does not include ltm-ConfigComplete);

NOTE 2:	When ltm-ConfigComplete is not included for an LTM candidate configuration, before an LTM cell switch is triggered a UE implementation may generate and store an RRCReconfiguration message by applying the received LTM candidate configuration on top of the LTM reference configuration, and the stored RRCReconfiguration message is applied when the LTM cell switch is triggered. It is up to the UE to ensure that the RRC reconfiguration applied at the time of LTM cell switch is in accordance with the latest LTM reference configuration and LTM candidate configuration.

[TS 38.331, clause 5.3.1.2]

For a UE provided with an sk-counter, keyToUse indicates whether the UE uses the master key (KgNB) or the secondary key (S-KeNB or S-KgNB) for a particular DRB. The secondary key is derived from the master key and sk-Counter, as defined in TS 33.501[11]. Whenever there is a need to refresh the secondary key, e.g. upon change of MN with KgNB change or to avoid COUNT reuse, the security key update is used (see 5.3.5.7). When the UE is in NR-DC, the network may provide a UE configured with an SCG with an sk-Counter even when no DRB is setup using the secondary key (S-KgNB) in order to allow the configuration of SRB3. The network can also provide the UE with an sk-Counter, even if no SCG is configured, when using SN terminated MCG bearers.

[TS 38.322, clause 5.1.2]

When upper layers request an RLC entity re-establishment, the UE shall:

-	discard all RLC SDUs, RLC SDU segments, and RLC PDUs, if any;

-	stop and reset all timers;

-	reset all state variables to their initial values.

[TS 38.322, clause 7.1]

This sub clause describes the state variables used in AM and UM entities in order to specify the RLC protocol. The state variables defined in this subclause are normative.

All state variables and all counters are non-negative integers.

All state variables related to AM data transfer can take values from 0 to 4095 for 12 bit SN or from 0 to 262143 for 18 bit SN. All arithmetic operations contained in the present document on state variables related to AM data transfer are affected by the AM modulus (i.e. final value = [value from arithmetic operation] modulo 4096 for 12 bit SN and 262144 for 18 bit SN).

All state variables related to UM data transfer can take values from 0 to 63 for 6 bit SN or from 0 to 4095 for 12 bit SN. All arithmetic operations contained in the present document on state variables related to UM data transfer are affected by the UM modulus (i.e. final value = [value from arithmetic operation] modulo 64 for 6 bit SN and 4096 for 12 bit SN).

When performing arithmetic comparisons of state variables or SN values, a modulus base shall be used.

TX_Next_Ack and RX_Next shall be assumed as the modulus base at the transmitting side and receiving side of an AM RLC entity, respectively. This modulus base is subtracted from all the values involved, and then an absolute comparison is performed (e.g. RX_Next <= SN < RX_Next + AM_Window_Size is evaluated as [RX_Next – RX_Next] modulo 2[sn-FieldLength] <= [SN – RX_Next] modulo 2[sn-FieldLength] < [RX_Next + AM_Window_Size – RX_Next] modulo 2[sn-FieldLength]), where sn-FieldLength is 12 or 18 for 12 bit SN and 18 bit SN, respectively.

RX_Next_Highest– UM_Window_Size shall be assumed as the modulus base at the receiving side of an UM RLC entity. This modulus base is subtracted from all the values involved, and then an absolute comparison is performed (e.g. (RX_Next_Highest– UM_Window_Size) <= SN < RX_Next_Highest is evaluated as [(RX_Next_Highest– UM_Window_Size) – (RX_Next_Highest– UM_Window_Size)] modulo 2[sn-FieldLength] <= [SN – (RX_Next_Highest– UM_Window_Size)] modulo 2[sn-FieldLength] < [RX_Next_Highest– (RX_Next_Highest– UM_Window_Size)] modulo 2[sn-FieldLength]), where sn-FieldLength is 6 or 12 for 6 bit SN and 12 bit SN, respectively.

The transmitting side of each AM RLC entity shall maintain the following state variables:

a) TX_Next_Ack – Acknowledgement state variable

This state variable holds the value of the SN of the next RLC SDU for which a positive acknowledgment is to be received in-sequence, and it serves as the lower edge of the transmitting window. It is initially set to 0, and is updated whenever the AM RLC entity receives a positive acknowledgment for an RLC SDU with SN = TX_Next_Ack.

b) TX_Next – Send state variable

This state variable holds the value of the SN to be assigned for the next newly generated AMD PDU. It is initially set to 0, and is updated whenever the AM RLC entity constructs an AMD PDU with SN = TX_Next and contains an RLC SDU or the last segment of a RLC SDU.

c) POLL_SN – Poll send state variable

This state variable holds the value of the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer when POLL_SN is set according to sub clause 5.3.3.2. It is initially set to 0.

The transmitting side of each AM RLC entity shall maintain the following counters:

a) PDU_WITHOUT_POLL – Counter

This counter is initially set to 0. It counts the number of AMD PDUs sent since the most recent poll bit was transmitted.

b) BYTE_WITHOUT_POLL – Counter

This counter is initially set to 0. It counts the number of data bytes sent since the most recent poll bit was transmitted.

c) RETX_COUNT – Counter

This counter counts the number of retransmissions of an RLC SDU or RLC SDU segment (see subclause 5.3.2). There is one RETX_COUNT counter maintained per RLC SDU.

The receiving side of each AM RLC entity shall maintain the following state variables:

a) RX_Next – Receive state variable

This state variable holds the value of the SN following the last in-sequence completely received RLC SDU, and it serves as the lower edge of the receiving window. It is initially set to 0, and is updated whenever the AM RLC entity receives an RLC SDU with SN = RX_Next.

b) RX_Next_Status_Trigger – t-Reassembly state variable

This state variable holds the value of the SN following the SN of the RLC SDU which triggered t-Reassembly.

c) RX_Highest_Status – Maximum STATUS transmit state variable

This state variable holds the highest possible value of the SN which can be indicated by "ACK_SN" when a STATUS PDU needs to be constructed. It is initially set to 0.

d) RX_Next_Highest – Highest received state variable

This state variable holds the value of the SN following the SN of the RLC SDU with the highest SN among received RLC SDUs. It is initially set to 0.

Each transmitting UM RLC entity shall maintain the following state variables:

a) TX_Next

This state variable holds the value of the SN to be assigned for the next newly generated UMD PDU with segment. It is initially set to 0, and is updated after the UM RLC entity submits a UMD PDU including the last segment of an RLC SDU to lower layers.

Each receiving UM RLC entity shall maintain the following state variables and constant:

b) RX_Next_Reassembly – UM receive state variable

This state variable holds the value of the earliest SN that is still considered for reassembly. It is initially set to 0.

c) RX_Timer_Trigger – UM t-Reassembly state variable

This state variable holds the value of the SN following the SN which triggered t-Reassembly.

d) RX_Next_Highest– UM receive state variable

This state variable holds the value of the SN following the SN of the UMD PDU with the highest SN among received UMD PDUs. It serves as the higher edge of the reassembly window. It is initially set to 0.

7.1.2.3.12.3	Test description

7.1.2.3.12.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.2.1.1 with the exception except the following:

-	3 NR cells (NR Cell 1, NR Cell 10 and NR Cell 30) are configured. NR Cell 1 is the PCell, NR Cell 10 is the PSCell, neighbour cell NR Cell 30 is on the same frequency as the PSCell.

-	The UE is in state RRC_CONNECTED using generic procedure parameter Connectivity (NR-DC), Bearers (MCG(s) and SCG) established according to TS 38.508-1 [4], clause 4.5.4. The UE uses the secondary key for SCG DRB.

-	AM DRB is configured according to Table 7.1.2.3.12.3.1-1.

Table 7.1.2.3.12.3.1-1: RLC parameters

| Parameter | Value |
| --- | --- |
| t-Reassembly | ms200 |
| t-PollRetransmit | ms4000 |

7.1.2.3.12.3.2	Test procedure sequence

Tables 7.1.2.3.12.3.2-1 and 7.1.2.3.12.3.2-2 illustrate the downlink power levels to be applied for NR Cell 1, NR Cell 10 and NR Cell 30 at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while the configurations marked "T1", "T2" and "T3" are applied at the point indicated in the Main behaviour description in Table 7.1.2.3.12.3.2-3.

Table 7.1.2.3.12.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 10 | NR Cell 30 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -85 | -88 | -94 | Power levels are such that entry condition for event A3 is not satisfied for the neighbour NR cell |
| T1 | SS/PBCH SSS EPRE | dBm/ SCS | -85 | -88 | -82 | Power levels are such that entry condition for event A3 is satisfied for intra-frequency neighbour NR cell |
| T2 | SS/PBCH SSS EPRE | dBm/ SCS | -85 | -88 | -94 | Power levels are such that entry condition for event A3 is satisfied for intra-frequency neighbour NR cell |
| T3 | SS/PBCH SSS EPRE | dBm/ SCS | -85 | -88 | -82 | Power levels are such that entry condition for event A3 is satisfied for intra-frequency neighbour NR cell |

Table 7.1.2.3.12.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 10 | NR Cell 30 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -88 | -91 | -100 | Power levels are such that entry condition for event A3 is not satisfied for the neighbour NR cell |
| T1 | SS/PBCH SSS EPRE | dBm/ SCS | -88 | -91 | -82 | Power levels are such that entry condition for event A3 is satisfied for intra-frequency neighbour NR cell |
| T2 | SS/PBCH SSS EPRE | dBm/ SCS | -88 | -91 | -100 | Power levels are such that entry condition for event A3 is satisfied for intra-frequency neighbour NR cell |
| T3 | SS/PBCH SSS EPRE | dBm/ SCS | -88 | -91 | -82 | Power levels are such that entry condition for event A3 is satisfied for intra-frequency neighbour NR cell |

Table 7.1.2.3.12.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits one AMD PDU containing SDU#1 (91 bytes) in its data field on NR Cell 10. | <-- | AMD PDU#1 | - | - |
| 2 | The UE transmits one AMD PDU containing SDU#1 in its data field. | --> | AMD PDU#1 (SN=0) | - | - |
| 3 | The SS transmits an RRCReconfiguration message on NR Cell 1 containing an RRCReconfiguration message including MeasConfig to setup NR measurement and reporting for intra-frequency event A3. | <-- | RRCReconfiguration (RRCReconfiguration) | - | - |
| 4 | The UE transmits an RRCReconfigurationComplete message containing RRCReconfigurationComplete message on NR Cell 1. | --> | RRCReconfigurationComplete (RRCReconfigurationComplete) | - | - |
| 5 | The SS creates 1 RLC SDU of size 40 bytes segmented into two PDUs (AMD PDU#2 and AMD PDU#3). SS transmits AMD PDU#3 (SN=1) on NR Cell 10. | <-- | AMD PDU#3 (SN=1) | - | - |
| 6 | SS re-adjusts the cell-specific reference signal level according to row "T1" in table 7.1.2.3.12.3.2-1/2. | - | - | - | - |
| 7 | The UE transmits a MeasurementReport message embedded in NR RRC message ULInformationTransferMRDC to report event A3 with the measured RSRP for NR Cell 30 on NR Cell 1. | --> | ULInformationTransferMRDC (MeasurementReport) | - | - |
| 8 | The SS transmits an RRCReconfiguration message on NR Cell 1 containing an RRCReconfiguration message with ltm-NoResetID absent in LTM-Candidate-r18 for NR Cell 30. | <-- | RRCReconfiguration (RRCReconfiguration) | - | - |
| 9 | The UE transmits an RRCReconfigurationComplete message containing RRCReconfigurationComplete message on NR Cell 1. | --> | RRCReconfigurationComplete (RRCReconfigurationComplete) | - | - |
| 10 | The SS transmits the candidate cell TCI activation MAC CE with Candidate Cell ID set to 0 for NR Cell 30 on NR Cell 10. | <-- | MAC PDU (Candidate Cell TCI state activation/deactivation MAC CE) | - | - |
| 11 | The SS transmits DCI 1_0 PDCCH order to trigger the UE PRACH transmission on NR Cell 10. | <-- | (PDCCH Order) | - | - |
| 12 | The UE transmits the PRACH preamble on NR Cell 30. | --> | (PRACH Preamble) | - | - |
| 13 | The SS transmits LTM cell switch command MAC CE with Timing Advance Command value set to 0 and Target Configuration ID value set to 0 on NR Cell 10 to the UE. | <-- | MAC PDU (LTM Cell Switch Command MAC CE) | - | - |
| 14 | The UE transmits an ULInformationTransferMRDC message containing RRCReconfigurationComplete message on NR Cell 1. | --> | ULInformationTransferMRDC (RRCReconfigurationComplete) | - | - |
| 15 | SS transmits AMD PDU#2 with SN=0 and the P field set to "1" on NR Cell 30. | <-- | AMD PDU#2 | - | - |
| 16 | Check: For 1 second, does the UE return RLC SDU? | --> | (RLC SDU) | 1 | F |
| 17 | SS transmits AMD PDU#3 with SN=0 on NR Cell 30. | <-- | AMD PDU#3 | - | - |
| - | EXCEPTION: RLC PDUs at steps 18 and 19 may be transmitted in the same slot or in multiple slots. | - | - | - | - |
| 18 | UE transmits a STATUS PDU (ACK_SN=0) on NR Cell 30. | --> | STATUS PDU | - | - |
| 19 | Check: Does the UE return RLC SDU with SN=0 on NR Cell 30? | --> | (RLC SDU) | 1 | P |
| 20 | SS transmits a STATUS PDU (ACK_SN = 0) NR Cell 30. | <-- | STATUS PDU | - | - |
| 21 | The SS transmits an RRCReconfiguration message on NR Cell 1 containing an RRCReconfiguration message including MeasConfig to setup NR measurement and reporting for intra-frequency event A3. | <-- | RRCReconfiguration (RRCReconfiguration) | - | - |
| 22 | The UE transmits an RRCReconfigurationComplete message containing RRCReconfigurationComplete message on NR Cell 1. | --> | RRCReconfigurationComplete (RRCReconfigurationComplete) | - | - |
| 23 | The SS creates 1 RLC SDU of size 40 bytes segmented into two PDUs (AMD PDU#4 and AMD PDU#5). SS transmits AMD PDU#5 (SN=1) on NR Cell 30. | <-- | AMD PDU#5(SN=1) | - | - |
| 24 | SS re-adjusts the cell-specific reference signal level according to row "T2" in table 7.1.2.3.12.3.2-1/2. | - | - | - | - |
| 25 | The UE transmits a MeasurementReport message embedded in NR RRC message ULInformationTransferMRDC to report event A3 with the measured RSRP for NR Cell 10 on NR Cell 1. | --> | ULInformationTransferMRDC (MeasurementReport) | - | - |
| 26 | The SS transmits an RRCReconfiguration message on NR Cell 1 containing an RRCReconfiguration message with ltm-NoResetID not equal to the value of ltm-ServingCellNoResetID for NR Cell 10. | <-- | RRCReconfiguration (RRCReconfiguration) | - | - |
| 27 | The UE transmits an RRCReconfigurationComplete message containing RRCReconfigurationComplete message on NR Cell 1. | --> | RRCReconfigurationComplete (RRCReconfigurationComplete) | - | - |
| 28 | The SS transmits the candidate cell TCI activation MAC CE with Candidate Cell ID set to 0 for NR Cell 10 on NR Cell 30. | <-- | MAC PDU (Candidate Cell TCI state activation/deactivation MAC CE) | - | - |
| 29 | The SS transmits DCI 1_0 PDCCH order to trigger the UE PRACH transmission on NR Cell 30. | <-- | (PDCCH Order) | - | - |
| 30 | The UE transmits the PRACH preamble on NR Cell 10. | --> | (PRACH Preamble) | - | - |
| 31 | The SS transmits LTM cell switch command MAC CE with Timing Advance Command value set to 0 and Target Configuration ID value set to 0 on NR Cell 30 to the UE. | <-- | MAC PDU (LTM Cell Switch Command MAC CE) | - | - |
| 32 | The UE transmits an ULInformationTransferMRDC message containing RRCReconfigurationComplete message on NR Cell 1. | --> | ULInformationTransferMRDC (RRCReconfigurationComplete) | - | - |
| 33 | SS transmits AMD PDU#4 with SN=0 and the P field set to "1" on NR Cell 10. | <-- | AMD PDU#4 | - | - |
| 34 | Check: For 1 second, does the UE return RLC SDU? | --> | (RLC SDU) | 2 | F |
| 35 | SS transmits AMD PDU#5 with SN=0 on NR Cell 10. | <-- | AMD PDU#5 | - | - |
| - | EXCEPTION: RLC PDUs at steps 36 and 37 may be transmitted in the same slot or in multiple slots. | - | - | - | - |
| 36 | UE transmits a STATUS PDU (ACK_SN=0) on NR Cell 10. | --> | STATUS PDU | - | - |
| 37 | Check: Does the UE return RLC SDU with SN=0 on NR Cell 10? | --> | (RLC SDU) | 2 | P |
| 38 | SS transmits a STATUS PDU (ACK_SN = 0) on NR Cell 10. | <-- | STATUS PDU | - | - |
| 39 | The SS transmits an RRCReconfiguration message on NR Cell 1 containing an RRCReconfiguration message including MeasConfig to setup NR measurement and reporting for intra-frequency event A3. | <-- | RRCReconfiguration (RRCReconfiguration) | - | - |
| 40 | The UE transmits an RRCReconfigurationComplete message containing RRCReconfigurationComplete message on NR Cell 1. | --> | RRCReconfigurationComplete (RRCReconfigurationComplete) | - | - |
| 41 | The SS creates 1 RLC SDU of size 40 bytes segmented into two PDUs (AMD PDU#6 and AMD PDU#7). SS transmits AMD PDU#7(SN=1) on NR Cell 10. | <-- | AMD PDU#7 (SN=1) | - | - |
| 42 | SS re-adjusts the cell-specific reference signal level according to row "T3" in table 7.1.2.3.12.3.2-1/2. | - | - | - | - |
| 43 | The UE transmits a MeasurementReport message embedded in NR RRC message ULInformationTransferMRDC to report event A3 with the measured RSRP for NR Cell 30 on NR Cell 1. | --> | ULInformationTransferMRDC (MeasurementReport) | - | - |
| 44 | The SS transmits an RRCReconfiguration message on NR Cell 1 containing an RRCReconfiguration message with ltm-NoResetID equal to the value of ltm-ServingCellNoResetID for NR Cell 30. | <-- | RRCReconfiguration (RRCReconfiguration) | - | - |
| 45 | The UE transmits an RRCReconfigurationComplete message containing RRCReconfigurationComplete message on NR Cell 1. | --> | RRCReconfigurationComplete (RRCReconfigurationComplete) | - | - |
| 46 | The SS transmits the candidate cell TCI activation MAC CE with Candidate Cell ID set to 0 for NR Cell 30 on NR Cell 10. | <-- | MAC PDU (Candidate Cell TCI state activation/deactivation MAC CE) | - | - |
| 47 | The SS transmits DCI 1_0 PDCCH order to trigger the UE PRACH transmission on NR Cell 10. | <-- | (PDCCH Order) | - | - |
| 48 | The UE transmits the PRACH preamble on NR Cell 30. | --> | (PRACH Preamble) | - | - |
| 49 | The SS transmits LTM cell switch command MAC CE with Timing Advance Command value set to 0 and Target Configuration ID value set to 0 on NR Cell 10 to the UE. | <-- | MAC PDU (LTM Cell Switch Command MAC CE) | - | - |
| 50 | The UE transmits an ULInformationTransferMRDC message containing RRCReconfigurationComplete message on NR Cell 1. | --> | ULInformationTransferMRDC (RRCReconfigurationComplete) | - | - |
| 51 | SS transmits AMD PDU#6 with SN=1 and the P field set to "1" on NR Cell 30. | <-- | AMD PDU#2 | - | - |
| - | EXCEPTION: RLC PDUs at steps 53 and 54 may be transmitted in the same slot or in multiple slots. | - | - | - | - |
| 52 | UE transmits a STATUS PDU (ACK_SN=1) NR Cell 30. | --> | STATUS PDU | - | - |
| 53 | Check: Does the UE return RLC SDU with SN=1 NR Cell 30? | --> | (RLC SDU) | 3 | P |
| 54 | SS transmits a STATUS PDU (ACK_SN = 1) NR Cell 30. | <-- | STATUS PDU | - | - |

7.1.2.3.12.3.3	Specific message contents

Table 7.1.2.3.12.3.3-1: RRCReconfiguration (Preamble, Table 7.1.2.3.12.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| mrdc-SecondaryCellGroupConfig CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| mrdc-SecondaryCellGroup CHOICE { |  |  |  |
| nr-SCG | OCTET STRING (CONTAINING RRCReconfiguration) as defined in TS 38.508-1 [4], Table 4.6.1-13 with condition NR-DC_SCG |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| radioBearerConfig2 | OCTET STRING (CONTAINING RadioBearerConfig) as defined in TS 38.508-1 [4], with condition DRBn and SecondaryKeys |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.2.3.12.3.3-2: RRCReconfiguration (step 3, step 21, step 39, Table 7.1.2.3.12.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with conditions NR_MEAS and NR-DC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| mrdc-SecondaryCellGroupConfig CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| mrdc-SecondaryCellGroup CHOICE { |  |  |  |
| nr-SCG | OCTET STRING (CONTAINING RRCReconfiguration as defined Table 7.1.2.3.12.3.3-8) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| radioBearerConfig2 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.2.3.12.3.3-3: MeasConfig (Table 7.1.2.3.12.3.3-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR | Table 7.1.2.3.12.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-EventA3 | Table 7.1.2.3.12.3.3-5 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.2.3.12.3.3-4: MeasObjectNR (Table 7.1.2.3.12.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR ::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for SSB of NR Cell 10 |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| nrofSS-BlocksToAverage | Not present |  |  |
| } |  |  |  |

Table 7.1.2.3.12.3.3-5: ReportConfigNR-EventA3 (Table 7.1.2.3.12.3.3-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-142 with condition EVENT_A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | 2 | 1 dB(2*0.5 dB) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportAmount | infinity |  |  |
| maxNrofRS-IndexesToReport | 1 |  |  |
| includeBeamMeasurements | true |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | true |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.2.3.12.3.3-6: MeasurementReport (step 7, step 25, step 43, Table 7.1.2.3.12.3.2-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-5A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| measurementReport SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServingMOList SEQUENCE (SIZE (1..maxNrofServingCells)) OF MeasResultServMO { | 1 entry |  |  |
| MeasResultServMO[1] SEQUENCE { |  | entry 1 |  |
| servCellId | ServCellIndex of NR Cell 10 |  | Step 7, Step 43 |
|  | ServCellIndex of NR Cell 30 |  | Step 25 |
| measResultServingCell SEQUENCE { |  |  |  |
| physCellId | PCI of NR Cell 10 |  | Step 7, Step 43 |
|  | PCI of NR Cell 30 |  | Step 25 |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not present |  |  |
|  | Not checked |  | pc_ss_SINR_Meas |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListNR SEQUENCE (SIZE (1.. maxCellReport)) OF MeasResultNR { | 1 entry |  |  |
| MeasResultNR[1] SEQUENCE { |  | entry 1 |  |
| physCellId | Physical layer cell identity of NR Cell 30 |  | Step 7, Step 43 |
|  | Physical layer cell identity of NR Cell 10 |  | Step 25 |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | Not present |  |  |
| sinr | Not present |  |  |
| } |  |  |  |
| resultsCSI-RS-Cell | Not present |  |  |
| } |  |  |  |
| rsIndexResults | Not present |  |  |
| } |  |  |  |
| cgi-Info | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.2.3.12.3.3-7: RRCReconfiguration (step 8, step 26, step 44, Table 7.1.2.3.12.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| mrdc-SecondaryCellGroupConfig CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| mrdc-SecondaryCellGroup CHOICE { |  |  |  |
| nr-SCG | RRCReconfiguration | Table 7.1.2.3.12.3.3-8 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.2.3.12.3.3-8: RRCReconfiguration (Table 7.1.2.3.12.3.2-7)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ltm-Config-r18 CHOICE { |  |  |  |
| setup | LTM-Config-r18 | Table 7.1.2.3.12.3.3-9 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.2.3.12.3.3-9: LTM-Config-r18(Table 7.1.2.3.12.3.3-8)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-67D |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-Config-r18 ::= SEQUENCE { |  |  |  |
| ltm-CandidateToAddModList-r18 SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-Candidate-r18 { | 1 entry |  |  |
| LTM-Candidate-r18[1] | LTM-Candidate-r18 | Table 7.1.2.3.12.3.3-10 |  |
| } |  |  |  |
| ltm-ServingCellNoResetID-r18 | Not present |  | Step 8 |
|  | 2 |  | Step 26 |
|  | 1 |  | Step 44 |
| ltm-CSI-ResourceConfig-r18 | Not present |  |  |
| } |  |  |  |

Table 7.1.2.3.12.3.3-10: LTM-Candidate-r18 (Table 7.1.2.3.12.3.3-9)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-67C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-Candidate-r18 ::= SEQUENCE { |  |  |  |
| ltm-CandidateId-r18 | 1 |  |  |
| ltm-CandidatePCI-r18 | Physical layer cell identity of NR Cell 30 |  | Step 8 Step 44 |
|  | Physical layer cell identity of NR Cell 10 |  | Step 26 |
| ltm-CandidateConfig-r18 OCTET STRING (CONTAINING RRCReconfiguration) { |  |  |  |
| RRCReconfiguration SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| secondaryCellGroup | CellGroupConfig specified in TS 38.508-1 [4] Table 4.6.3-19 with condition NR-DC_SCG |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ltm-TCI-Info-r18 | LTM-TCI-Info specified in TS 38.508-1 [4] Table 4.6.3-67I with condition Joint |  |  |
| ltm-NoResetID-r18 | Not present |  | Step 8 |
|  | 1 |  | Step 26, Step 44 |
| } |  |  |  |

### 7.1.3	PDCP

#### 7.1.3.0	Default Pre-Test Conditions for all PDCP test cases

The following pre-test conditions shall be applied in all PDCP test cases until the test case explicitly over writes these conditions

System Simulator:

-	The SS configures the test environment in accordance to the execution conditions in Table 7.1.3.0-1.

UE:

-	None

Preamble:

-	The SS performs the generic procedure in [4] to get UE in state RRC_CONNECTED in accordance to the execution conditions in Table 7.1.3.0-2 and using the message condition UE TEST LOOP MODE A to return one UL PDCP SDU per DL PDCP SDU.

Table 7.1.3.0-1: Test environment

| Execution Condition | Cell configuration | System Information Combination |
| --- | --- | --- |
| IF pc_NG_RAN_NR and Connectivity(NR-DC) | NR Cell 1 is PCell NR Cell 10 is PSCell | NR System information Combination NR-1 |
| IF pc_NG_RAN_NR | NR Cell 1 | NR System information Combination NR-1 |
| ELSE IF pc_EN_DC | E-UTRA Cell 1 is PCell, NR Cell 1 is PSCell | EUTRA: System information Combination 1 NR: N/A |
| ELSE IF pc_NGEN_DC | NG-RAN E-UTRA Cell 1 is PCell, NR Cell 1 is PSCell | EUTRA: System information Combination 1 NR: N/A |
| ELSE IF pc_NE_DC | NR Cell 1 is PCell, E-UTRA Cell 1 is PSCell | NR: System information Combination NR-1 EUTRA: N/A |

Table 7.1.3.0-2: Preamble parameters

| Execution Condition | Multi-PDN / Multi-PDU Sessions Condition | Generic Procedure Parameters | Primary DRB used for Data testing |
| --- | --- | --- | --- |
| IF pc_NG_RAN_NR | FALSE | Connectivity(NR), Test loop function(On) One DRB | Default DRB of the first PDU session on NR Cell |
|  |  | Connectivity(NR-DC), DC bearer(One MN Terminated MCG bearer and One SN Terminated SCG bearer), Test loop function(On) | SN terminated SCG DRB |
|  | TRUE | Connectivity(NR), Test loop function(On) N DRBs (N ≥ 2) | Default DRB of the first PDU session on NR Cell |
|  |  | Connectivity(NR-DC), DC bearer(N MN Terminated MCG bearers and One SN Terminated SCG bearer), N DRBs (N ≥ 2) Test loop function(On) | SN terminated SCG DRB |
| ELSE IF pc_EN_DC | FALSE | Connectivity(EN-DC), DC bearer(One MN Terminated MCG bearer and One SN terminated SCG bearer), Test loop function(On) | SN Terminated SCG bearer unless explicitly specified in test case |
|  | TRUE | Connectivity(EN-DC), DC bearer(Two MN Terminated MCG bearer and One SN terminated SCG bearer), Test loop function(On) |  |
| ELSE IF pc_NGEN_DC | FALSE | Connectivity(NGEN-DC), DC bearer(One MN Terminated MCG bearer and One SN terminated SCG bearer), Test loop function(On) | SN Terminated SCG bearer unless explicitly specified in test case |
|  | TRUE | Connectivity(NGEN-DC), DC bearer(Two MN Terminated MCG bearer and One SN terminated SCG bearer), Test loop function(On) |  |
| ELSE IF pc_NE_DC | FALSE | Connectivity(NE-DC), DC bearer(One MN Terminated MCG bearer and One SN terminated SCG bearer), Test loop function(On) | SN Terminated SCG bearer unless explicitly specified in test case |
|  | TRUE | Connectivity(NE-DC), DC bearer(N ≥ 2 MN Terminated MCG bearer and One SN terminated SCG bearer), Test loop function(On) |  |

Table 7.1.3.0-3: Message conditions

| Execution Condition | Message condition exceptions |
| --- | --- |
| IF pc_NG_RAN_NR | Message with condition AM is used for step 7 in 4.5.4.2 according to [4] |
| ELSE IF pc_EN_DC | Message condition MCG_and_SCG with condition AM is used for step 7 in 4.5.4.2 according to [4] |
| ELSE IF pc_NGEN_DC | Message condition MCG_and_SCG with condition AM is used for step 7 in 4.5.4.2 according to [4] |
| ELSE IF pc_NE_DC | Message condition MCG_and_SCG with condition AM is used for step 7 in 4.5.4.6 according to [4] |

#### 7.1.3.1	Maintenance of PDCP sequence numbers for radio bearers

##### 7.1.3.1.1	Maintenance of PDCP sequence numbers / User plane / 12 bit SN

7.1.3.1.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with PDCP configured for 12 bit SN}

ensure that {

when { UE transmits a PDCP Data SDU on a DRB }

then { UE increments SN with 1 for each transmitted PDU for SN=0 to Maximum_PDCP_SN (2[pdcp-SN-SizeUL] -1) }

}

(2)

with { UE in RRC_CONNECTED state with PDCP configured for 12 bit SN }

ensure that {

when { UE transmits a PDCP Data SDU on a DRB and, after incrementation, TX_NEXT is larger than the Maximum_PDCP_SN(2[pdcp-SN-SizeUL] -1) }

then { UE sets SN to 0 in the next transmitted PDCP SDU}

}

7.1.3.1.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.323, clauses 5.2.1, 5.2.2.1 and 6.2.2.2. Unless otherwise stated these are Rel-15 requirements.

[TS 38.323, clause 5.2.1]

At reception of a PDCP SDU from upper layers, the transmitting PDCP entity shall:

-	start the discardTimer associated with this PDCP SDU (if configured).

For a PDCP SDU received from upper layers, the transmitting PDCP entity shall:

-	associate the COUNT value corresponding to TX_NEXT to this PDCP SDU;

NOTE 1:	Associating more than half of the PDCP SN space of contiguous PDCP SDUs with PDCP SNs, when e.g., the PDCP SDUs are discarded or transmitted without acknowledgement, may cause HFN desynchronization problem. How to prevent HFN desynchronization problem is left up to UE implementation.

-	perform header compression of the PDCP SDU as specified in the subclause 5.7.4;

-	perform integrity protection, and ciphering using the TX_NEXT as specified in the subclause 5.9 and 5.8, respectively;

-	set the PDCP SN of the PDCP Data PDU to TX_NEXT modulo 2[pdcp-SN-Size];

-	increment TX_NEXT by one;

-	submit the resulting PDCP Data PDU to lower layer as specified below.

When submitting a PDCP Data PDU to lower layer, the transmitting PDCP entity shall:

-	if the transmitting PDCP entity is associated with one RLC entity:

-	submit the PDCP Data PDU to the associated RLC entity.

-	else, if the transmitting PDCP entity is associated with two RLC entities:

-	if pdcpDuplication is configured and activated:

-	duplicate the PDCP Data PDU and submit the PDCP Data PDU to both associated RLC entities.

-	else, if pdcpDuplication is configured but not activated:

-	submit the PDCP Data PDU to the primary RLC entity.

-	else:

-	if the total amount of PDCP data volume and RLC data volume pending for initial transmission (as specified in TS 36.322 [5]) in the two associated RLC entities is less than ul-DataSplitThreshold:

-	submit the PDCP Data PDU to the primary RLC entity.

-	else:

-	submit the PDCP Data PDU to either the primary RLC entity or the secondary RLC entity.

NOTE 2:	If the transmitting PDCP entity is associated with two RLC entities, the UE should minimize the amount of PDCP PDUs submitted to lower layers before receiving request from lower layers and minimize the PDCP SN gap between PDCP PDUs submitted to two associated RLC entities to minimize PDCP reordering delay in the receiving PDCP entity.

[TS 38.323, clause 5.2.2.1]

In this section, following definitions are used:

-	HFN(State Variable): the HFN part (i.e. the number of most significant bits equal to HFN length) of the State Variable;

-	SN(State Variable): the SN part (i.e. the number of least significant bits equal to PDCP SN length) of the State Variable;

-	RCVD_SN: the PDCP SN of the received PDCP Data PDU, included in the PDU header;

-	RCVD_HFN: the HFN of the received PDCP Data PDU, calculated by the receiving PDCP entity;

-	RCVD_COUNT: the COUNT of the received PDCP Data PDU = [RCVD_HFN, RCVD_SN]

At reception of a PDCP Data PDU from lower layers, the receiving PDCP entity shall determine the COUNT value of the received PDCP Data PDU, i.e. RCVD_COUNT, as follows:

-	if RCVD_SN < SN(RX_DELIV) – Window_Size:

-	RCVD_HFN = HFN(RX_DELIV) + 1.

-	else if RCVD_SN >= SN(RX_DELIV) + Window_Size:

-	RCVD_HFN = HFN(RX_DELIV) – 1.

-	else:

-	RCVD_HFN = HFN(RX_DELIV);

-	RCVD_COUNT = [RCVD_HFN, RCVD_SN].

After determining the COUNT value of the received PDCP Data PDU = RCVD_COUNT, the receiving PDCP entity shall:

-	if RCVD_COUNT < RX_DELIV; or

-	if the PDCP Data PDU with COUNT = RCVD_COUNT has been received before:

-	perform deciphering and integrity verification of the PDCP Data PDU using COUNT = RCVD_COUNT;

-	if integrity verification fails:

-	indicate the integrity verification failure to upper layer;

-	discard the PDCP Data PDU.

-	else:

-	perform deciphering and integrity verification of the PDCP Data PDU using COUNT = RCVD_COUNT;

-	if integrity verification fails:

-	indicate the integrity verification failure to upper layer;

-	discard the PDCP Data PDU.

If the received PDCP Data PDU with COUNT value = RCVD_COUNT is not discarded above, the receiving PDCP entity shall:

-	store the resulting PDCP SDU in the reception buffer;

-	if RCVD_COUNT >= RX_NEXT:

-	update RX_NEXT to RCVD_COUNT + 1.

-	if outOfOrderDelivery is configured:

-	deliver the resulting PDCP SDU to upper layers.

-	if RCVD_COUNT = RX_DELIV:

-	deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before;

-	all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from COUNT = RX_DELIV;

-	update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers, with COUNT value > RX_DELIV;

-	if t-Reordering is running, and if RX_DELIV >= RX_REORD:

-	stop and reset t-Reordering.

-	if t-Reordering is not running (includes the case when t-Reordering is stopped due to actions above), and RX_DELIV < RX_NEXT:

-	update RX_REORD to RX_NEXT;

-	start t-Reordering.

[TS 38.322, clause 6.2.2.2]

Figure 6.2.2.2-1 shows the format of the PDCP Data PDU with 12 bits PDCP SN. This format is applicable for UM DRBs and AM DRBs.

Figure 6.2.2.2-1: PDCP Data PDU format with 12 bits PDCP SN

7.1.3.1.1.3	Test description

7.1.3.1.1.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.3.0 exception of PDCP parameters according to Table 7.1.3.1.1.3.1-1.

Table 7.1.3.1.1.3.1-1: PDCP parameters

| PDCP-Config pdcp-SN-SizeUL | len12bits |
| --- | --- |
| PDCP-Config pdcp-SN-SizeDL | len12bits |

7.1.3.1.1.3.2	Test procedure sequence

Table 7.1.3.1.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 1 – 2 shall be repeated from j = 0 to j = FLOOR(Maximum_PDCP_SN/iteration_size). (Note 1, 4) | - | - | - | - |
| 1 | SS transmits in one slot, several PDCP PDUs in a PDCP PDU list without header compression, the number of PDCP PDUs sent in each PDCP PDU list is defined by the iteration_size. (Note 4, 5). | <-- | PDCP PDUs | - | - |
| - | EXCEPTION: In Step 2, SS may receive a PDCP PDU or several PDCP PDUs, then step 2 may be repeated multiple times until all PDCP PDUs with SN=j*iteration_size to SN=(((j+1)* iteration_size)-1) for each iteration are received. | - | - | - | - |
| 2 | Check: Does UE transmit PDCP PDUs with SN=0 for the first iteration and all PDCP PDUs for each iteration? (Note 2) (Note 6) | --> | PDCP PDUs | 1 | P |
| 3 | SS transmits a PDCP PDU containing one PDCP SDU without header compression. | <-- | PDCP PDU | - | - |
| 4 | Check: Does UE transmit a PDCP PDU with SN=0? | --> | PDCP PDU | 2 | P |
| 5 | SS transmits a PDCP PDU containing one PDCP SDU without header compression. | <-- | PDCP PDU | - | - |
| 6 | Check: Does UE transmit a PDCP PDU with SN=1? | --> | PDCP PDU | 1 | P |
| Note 1: Maximum_PDCP_SN = 2[pdcp-SN-SizeUL] -1. Note 2: The verdict shall be provided each time [(SN+1) mod 256 = 0] for 12 bit SN and [(SN+1) mod 4096 = 0] for 18 bit SN respectively. Note 3: Void Note 4: Iteration will be incremented by iteration_size of 1 for 12 bit SN and iteration_size of 25.for 18 bits SN. Small PDCP SDU size will be used. Note 5: SS shall transmit a PDCP PDU list with size equal to iteration_size and incrementing SN by 1 till SN = ((j+1)*iteration_size)-1. Note 6 All PDCP PDUs may be received by the SS in the same slot or in multiple slots (max one MAC PDU in a slot) |  |  |  |  |  |

7.1.3.1.1.3.3	Specific message contents

None.

##### 7.1.3.1.2	Maintenance of PDCP sequence numbers / User plane / 18 bit SN

7.1.3.1.2.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with PDCP configured for 18 bit SN}

ensure that {

when { UE transmits a PDCP Data SDU on a DRB }

then { UE increments SN with 1 for each transmitted PDU for SN=0 to Maximum_PDCP_SN (2[pdcp-SN-SizeUL] -1) }

}

(2)

with { UE in RRC_CONNECTED state with PDCP configured for 18 bit SN }

ensure that {

when { UE transmits a PDCP Data SDU on a DRB and, after incrementation, TX_Next is larger than the Maximum_PDCP_SN (2[pdcp-SN-SizeUL] -1) }

then { UE sets SN to 0 in the next transmitted PDCP SDU}

}

7.1.3.1.2.2	Conformance requirements

Same as conformance requirements in clause 7.1.3.1.1.2

7.1.3.1.2.3	Test description

7.1.3.1.2.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.3.0  exception of PDCP parameters according to Table 7.1.3.1.2.3.1-1.

Table 7.1.3.1.2.3.1-1: PDCP parameters

| PDCP-Config pdcp-SN-SizeUL | len18bits |
| --- | --- |
| PDCP-Config pdcp-SN-SizeDL | len18bits |

7.1.3.1.2.3.2	Test procedure sequence

Same as test procedure in clause 7.1.3.1.1.3.2

7.1.3.1.2.3.3	Specific message contents

None.

#### 7.1.3.2	PDCP integrity protection

##### 7.1.3.2.1	Integrity protection / Correct functionality of integrity algorithm SNOW3G / SRB / DRB

(1)

with { UE in RRC_CONNECTED state and SRB is configured with NR-PDCP}

ensure that {

when { Functionality of integrity algorithms with SNOW3G is taken into use on SRB }

then { UE performs correct integrity protection function in NR-PDCP entities associated with SRB }

}

(2)

with { UE in RRC_CONNECTED state and NOT EN-DC }

ensure that {

when { Functionality of integrity algorithms with SNOW3G is taken into use on DRB }

then { UE performs correct integrity protection function in PDCP entities associated with DRB }

}

(3)

with { UE in RRC_CONNECTED state and SRB3 is configured }

ensure that {

when {  message on SRB 3 is received and fails the integrity protection check }

then { UE transmits SCGFailureInformationNR message with failure type 'srb3-IntegrityFailure' }

}

NOTE:	TP2 (integrity on DRB) is not applicable to EN-DC as per 38.331 clause 6.3.2, the IE PDCP-Config.drb.integrityProtection is‘ Cond ConnectedTo5GC‘.

7.1.3.2.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.323, clauses 5.9, 5.2.2.1, TS 33.501 clauses 5.6.2, D.3.1 and TS 38.331 clauses5.7.3.1, 5.7.3.2, 5.7.3.3. Unless otherwise stated these are Rel-15 requirements.

[TS 38.323, clause 5.9]

The integrity protection function includes both integrity protection and integrity verification and is performed in PDCP, if configured. The data unit that is integrity protected is the PDU header and the data part of the PDU before ciphering. The integrity protection is always applied to PDCP Data PDUs of SRBs. The integrity protection is applied to PDCP Data PDUs of DRBs for which integrity protection is configured. The integrity protection is not applicable to PDCP Control PDUs.

The integrity protection algorithm and key to be used by the PDCP entity are configured by upper layers TS 38.331 [3] and the integrity protection method shall be applied as specified in TS 33.501 [6].

The integrity protection function is activated by upper layers TS 38.331 [3]. When security is activated, the integrity protection function shall be applied to all PDUs including and subsequent to the PDU indicated by upper layers TS 38.331 [3] for the downlink and the uplink, respectively.

NOTE:	As the RRC message which activates the integrity protection function is itself integrity protected with the configuration included in this RRC message, this message needs first be decoded by RRC before the integrity protection verification could be performed for the PDU in which the message was received.

For downlink and uplink integrity protection and verification, the parameters that are required by PDCP for integrity protection are defined in TS 33.501 [6] and are input to the integrity protection algorithm. The required inputs to the integrity protection function include the COUNT value, and DIRECTION (direction of the transmission: set as specified in TS 33.501 [6]). The parameters required by PDCP which are provided by upper layers TS 38.331 [3] are listed below:

-	BEARER (defined as the radio bearer identifier in TS 33.501 [6]. It will use the value RB identity –1 as in TS 38.331 [3]);

-	KEY (the integrity protection keys for the control plane and for the user plane are KRRCint and KUPint, respectively).

At transmission, the UE computes the value of the MAC-I field and at reception it verifies the integrity of the PDCP Data PDU by calculating the X-MAC based on the input parameters as specified above. If the calculated X-MAC corresponds to the received MAC-I, integrity protection is verified successfully.

[TS 38.323, clause 5.2.2.1]

At reception of a PDCP Data PDU from lower layers, the receiving PDCP entity shall determine the COUNT value of the received PDCP Data PDU, i.e. RCVD_COUNT, as follows:

-	if RCVD_SN < SN(RX_DELIV) – Window_Size:

-	RCVD_HFN = HFN(RX_DELIV) + 1.

-	else if RCVD_SN >= SN(RX_DELIV) + Window_Size:

-	RCVD_HFN = HFN(RX_DELIV) – 1.

-	else:

-	RCVD_HFN = HFN(RX_DELIV);

-	RCVD_COUNT = [RCVD_HFN, RCVD_SN].

After determining the COUNT value of the received PDCP Data PDU = RCVD_COUNT, the receiving PDCP entity shall:

-	perform deciphering and integrity verification of the PDCP Data PDU using COUNT = RCVD_COUNT;

-	if integrity verification fails:

-	indicate the integrity verification failure to upper layer;

-	discard the PDCP Data PDU;

-	if RCVD_COUNT < RX_DELIV; or

-	if the PDCP Data PDU with COUNT = RCVD_COUNT has been received before:

-	discard the PDCP Data PDU;

[TS 33.501, clause 5.6.2]

All Identifiers and names specified in the present subclause are for 5G.

Each Integrity Algorithm used for 5G will be assigned a 4-bit identifier. The following values for integrity algorithms are defined:

"00002"         NIA0		Null Integrity Protection algorithm;

"00012"         128-NIA1	128-bit SNOW 3G based algorithm;

"00102"         128-NIA2	128-bit AES based algorithm; and

"00112"         128-NIA3	128-bit ZUC based algorithm.

128-NIA1 is based on SNOW 3G (see TS35.215 [14]).

128-NIA2 is based on 128-bit AES [15] in CMAC mode [17].

128-NIA3 is based on 128-bit ZUC (see TS35.221 [18]).

Full details of the algorithms are specified in Annex D.

[TS 33.501, clause D.3.1.1]

The input parameters to the integrity algorithm are a 128-bit integrity key named KEY, a 32-bit COUNT, a 5-bit bearer identity called BEARER, the 1-bit direction of the transmission i.e. DIRECTION, and the message itself i.e. MESSAGE. The DIRECTION bit shall be 0 for uplink and 1 for downlink. The bit length of the MESSAGE is LENGTH.

Figure D.3.1.1-1 illustrates the use of the integrity algorithm NIA to authenticate the integrity of messages.

Figure D.3.1.1-1: Derivation of MAC-I/NAS-MAC (or XMAC-I/XNAS-MAC)

Based on these input parameters the sender computes a 32-bit message authentication code (MAC-I/NAS-MAC) using the integrity algorithm NIA. The message authentication code is then appended to the message when sent. For integrity protection algorithms, the receiver computes the expected message authentication code (XMAC-I/XNAS-MAC) on the message received in the same way as the sender computed its message authentication code on the message sent and verifies the data integrity of the message by comparing it to the received message authentication code, i.e. MAC-I/NAS-MAC.

[TS 38.331, clause 5.7.3.1]

Figure 5.7.3.1-1: SCG failure information

The purpose of this procedure is to inform EUTRAN or NR MN about an SCG failure the UE has experienced i.e. SCG radio link failure, e failure of SCG reconfiguration with sync, SCG configuration failure for RRC message on SRB3, SCG integrity check failure and exceeding the maximum uplink transmission timing difference.

[TS 38.331, clause 5.7.3.2]

A UE initiates the procedure to report SCG failures when SCG transmission is not suspended and when one of the following conditions is met:

1>	upon detecting radio link failure for the SCG, in accordance with subclause 5.3.10.3;

1>	upon reconfiguration with sync failure of the SCG, in accordance with subclause 5.3.5.9.3;

1>	upon SCG configuration failure, in accordance with subclause 5.3.5.9.2;

1>	upon integrity check failure indication from SCG lower layers, in accordance with subclause 5.3.5.9.1.

Upon initiating the procedure, the UE shall:

1>	suspend SCG transmission for all SRBs and DRBs;

1>	reset SCG-MAC;

1>	stop T304, if running;

1>	if the UE is operating in EN-DC:

2>	initiate transmission of the SCGFailureInformationNR message as specified in TS 36.331 [10, 5.6.13a].

[TS 38.331, clause 5.7.3.3]

The UE shall set the SCG failure type as follows:

...

1>	else, if the UE initiates transmission of the SCGFailureInformationNR message due to SRB3 IP check failure:

2>	set the failureType as srb3-IntegrityFailure;

7.1.3.2.1.3	Test description

7.1.3.2.1.3.1	Pre-test conditions

-	Same Pre-test conditions as in clause 7.1.3.0 with the exception that integrity protection algorithm ‘nia1  (SNOW3G)’ is configured.

-	For EN_DC, same Pre-test conditions as in clause 7.1.3.0 with the exception that integrity protection algorithms ‘nia1 (SNOW3G)’ and ‘eia1 (SNOW3G)’ are configured and without message condition UE TEST LOOP MODE A.

-	For EN_DC or NGEN_DC, RRCConnectionReconfiguration message including MobilityControlInfo IE is transmitted on E-UTRA Cell 1 to reconfigure SRB1 and SRB2 from E-UTRA PDCP to NR PDCP.

7.1.3.2.1.3.2	Test procedure sequence

Table 7.1.3.2.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Steps 1a1 to 1b2 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that takes place depending on a particular configuration. | - | - | - | - |
| 1a1 | IF Connectivity is EN-DC or NGEN-DC, the SS sends EUTRA RRC UECapabilityEnquiry message including RAT-Type eutra-nr to the UE integrity protected. | <-- | RRC: UECapabilityEnquiry | - | - |
| 1a2 | Check: Does the UE send a EUTRA RRC UECapabilityInformation message integrity protected? | --> | RRC: UECapabilityInformation | 1 | P |
| 1b1 | ELSE the SS sends NR RRC UECapabilityEnquiry message to the UE. | <-- | NR RRC: UECapabilityEnquiry | - | - |
| 1b2 | Check: Does the UE send a NR RRC UECapabilityInformation message? | --> | NR RRC: UECapabilityInformation | 1 | P |
| - | EXCEPTION: Steps 2a1-2a4 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that takes place if SRB3 is configured | - | - | - | - |
| 2a1 | IF Connectivity is EN-DC or NGEN-DC, IF pc_srb3 then the SS transmits an RRCReconfiguration message to reconfigure NR MAC, sent on SRB3 integrity protected. Note 1 | <-- | RRCReconfiguration | - | - |
| 2a2 | Check: Does the UE transmit an RRCReconfigurationComplete message on SRB3 integrity protected? | --> | RRCReconfigurationComplete | 1 | P |
| 2a3 | The SS sends RRCReconfiguration message to the UE integrity protected on SRB3. The MAC-I is corrupted so as to result in integrity failure at UE. | <-- | RRCReconfiguration | - | - |
| 2a4 | Check: Does the UE send SCGFailureInformationNR with failureType ‘srb3-IntegrityFailure’ on SRB1? | --> | SCGFailureInformationNR | 3 | P |
| - | EXCEPTION: Steps 3a1-3a2 describe behaviour that depends on whether 5GC is being emulated; the "lower case letter" identifies a step sequence that takes place if 5GC is being emulated. | - | - | - | - |
| 3a1 | SS transmits PDCP PDU on DRB integrity protected. | <-- | PDCP PDU | - | - |
| 3a2 | Check: Does the UE transmit looped back PDCP PDU integrity protected on DRB? | --> | PDCP PDU | 2 | P |
| Note 1: For EN-DC the NR RRCReconfiguration is contained in RRCConnectionReconfiguration Table 7.1.3.2.1.3.3-1 |  |  |  |  |  |

7.1.3.2.1.3.3	Specific message contents

Table 7.1.3.2.1.3.3-1: RRCConnectionReconfiguration (Preamble for EN-DC or NGEN-DC)

| Derivation Path: 36.508 [7] Table 4.6.1-8 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  |  | Value/remark |  | Comment |  | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |  |  |  |
| rrcConnectionReconfiguration-r8 ::= SEQUENCE { |  |  |  |  |  |  |  |
| mobilityControlInfo |  |  | MobilityControlInfo-HO-SameCell |  | As per Table 7.1.3.2.1.3.3-1A |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |  |  |  |
| nonCriticalExtension ::= SEQUENCE { |  |  |  |  |  |  |
| nr-Config-r15 CHOICE { |  |  |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |  |  |
| nr-SecondaryCellGroupConfig-r15 | OCTET STRING including the RRCReconfiguration message according to TS 38.508-1 [4], table 4.6.1-13 with condition EN-DC_HO |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| sk-Counter-r15 | Increment the value by 1 from the previous value |  |  |  |  |  |
| nr-RadioBearerConfig1-r15 | OCTET STRING including RadioBearerConfig according to TS 38.508-1 [4], Table 4.6.3-132 with conditions EN-DC_DRB and Re-establish_PDCP |  |  |  |  |  |
| nr-RadioBearerConfig1-r15 | OCTET STRING including RadioBearerConfig according to TS 38.508-1 [4], Table 4.6.3-132 with conditions EN-DC_DRB and Re-establish_PDCP and SRB3 |  |  |  | SRB3 |  |
| nr-RadioBearerConfig2-r15 | OCTET STRING including RadioBearerConfig according to TS 38.508-1 [4], table 4.6.3-132 with condition SRB_NR_PDCP |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| SRB3 | Establishment of SRB3 |

Table 7.1.3.2.1.3.3-1A: MobilityControlInfo-HO-SameCell (Table 7.1.3.2.1.3.3-1)

| Derivation Path: 36.508 [7], Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo-HO ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of E-UTRA Cell 1 |  |  |
| carrierFreq | Not present |  |  |
| } |  |  |  |

Table 7.1.3.2.1.3.3-2: MAC-CellGroupConfig (step 2a1, Table 7.1.3.2.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-68 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MAC-CellGroupConfig ::= SEQUENCE { |  |  |  |
| bsr-Config SEQUENCE { |  |  |  |
| periodicBSR-Timer | sf10 | Different from default |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.2.1.3.3-3: SCGFailureInformationNR message (step 2a4, Table 7.1.3.2.1.3.2-1)

| Derivation Path: 36.508 [7], Table 4.6.1-18AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SCGFailureInformationNR-r15::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| scgFailureInformationNR-r15 SEQUENCE { |  |  |  |
| failureReportSCG-NR-r15 SEQUENCE { |  |  |  |
| failureType-r15 | srb3-IntegrityFailure |  |  |
| measResultFreqListNR-r15 | Not checked |  |  |
| measResultSCG-r15 | Not checked |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE {} |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.2.1.3.3-4: RRCReconfiguration (Preamble for NR/5GC)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig-IntegrityOnDRB |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.3.2.1.3.3-5: RadioBearerConfig-IntegrityOnDRB (Preamble for NR/5GC)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-132 with conditions SRB2 and DRB1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry | Primary DRB as per Table 7.1.3.0-2 | DRB1 |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config | SDAP-Config |  |  |
| } |  |  |  |
| drb-Identity | DRB-Identity using condition DRB1 |  |  |
| reestablishPDCP | Not present |  |  |
| recoverPDCP | Not present |  |  |
| pdcp-Config | PDCP-Config-IntegrityOnDRB |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.2.1.3.3-6: PDCP-Config-IntegrityOnDRB (Preamble for NR/5GC)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-99 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-Config ::= SEQUENCE { |  |  |  |
| drb SEQUENCE { |  |  |  |
| discardTimer | infinity |  |  |
| pdcp-SN-Size-UL | len18bits |  |  |
| pdcp-SN-Size-DL | len18bits |  |  |
| headerCompression CHOICE { |  |  |  |
| notUsed | NULL |  |  |
| } |  |  |  |
| integrityProtection | enabled |  |  |
| statusReportRequired | true |  |  |
| outOfOrderDelivery | Not present |  |  |
| } |  |  |  |
| moreThanOneRLC | Not present |  |  |
| t-Reordering | Not present |  |  |
| } |  |  |  |

##### 7.1.3.2.2	Integrity protection / Correct functionality of integrity algorithm AES / SRB / DRB

7.1.3.2.2.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and SRB is configured with NR-PDCP }

ensure that {

when { Functionality of integrity algorithms with AES is taken into use on SRB }

then { UE performs correct integrity protection function in NR-PDCP entity associated with SRB }

}

(2)

with { UE in RRC_CONNECTED state and NOT EN-DC }

ensure that {

when { Functionality of integrity algorithms with AES is taken into use on DRB }

then { UE performs correct integrity protection function in PDCP entities associated with DRB }

}

(3)

with { UE in RRC_CONNECTED state and SRB3 is configured }

ensure that {

when { message on SRB 3 is received and fails the integrity protection check }

then { UE transmits SCGFailureInformationNR message with failure type as srb3-IntegrityFailure }

}

NOTE:	TP2 (integrity on DRB) is not applicable to EN-DC as per TS 38.331 [12] clause 6.3.2, the IE PDCP-Config.drb.integrityProtection is 'Cond ConnectedTo5GC'.

7.1.3.2.2.2	Conformance requirements

Same conformance requirements as in clause 7.1.3.2.1.2

7.1.3.2.2.3	Test description

7.1.3.2.2.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.3.2.1.3.1 except that integrity protection algorithm ‘nia2 (AES)’ and ‘eia2 (AES)’ is configured.

7.1.3.2.2.3.2	Test procedure sequence

Same test procedure sequence as in clause 7.1.3.2.1.3.2.

7.1.3.2.2.3.3	Specific message contents

Same specific message contents as in clause 7.1.3.2.1.3.3 except for integrity protection algorithm ‘nia2 (AES)’ and ‘eia2 (AES)’.

##### 7.1.3.2.3	Integrity protection / Correct functionality of integrity algorithm ZUC / SRB / DRB

(1)

with { UE in RRC_CONNECTED state and SRB is configured with NR-PDCP }

ensure that {

when { Functionality of integrity algorithms with ZUC is taken into use on SRB }

then { UE performs correct integrity protection function in NR-PDCP entities associated with SRB }

}

(2)

with { UE in RRC_CONNECTED state and NOT EN-DC }

ensure that {

when { Functionality of integrity algorithms with ZUC is taken into use on DRB }

then { UE performs correct integrity protection function in PDCP entities associated with DRB }

}

(3)

with { UE in RRC_CONNECTED state and SRB3 is configured }

ensure that {

when { message on SRB 3 is received and fails the integrity protection check }

then { UE transmits SCGFailureInformationNR message with failure type as srb3-IntegrityFailure }

}

NOTE:	TP2 (integrity on DRB) is not applicable to EN-DC as per TS 38.331 [12] clause 6.3.2, the IE PDCP-Config.drb.integrityProtection is 'Cond ConnectedTo5GC.

7.1.3.2.3.2	Conformance requirements

Same conformance requirements as in clause 7.1.3.2.1.2.

7.1.3.2.3.3	Test description

7.1.3.2.3.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.3.2.1.3.1 except that integrity protection algorithm ‘nia3 (ZUC)’ and ‘eia3 (ZUC)’ is configured.

7.1.3.2.3.3.2	Test procedure sequence

Same test procedure sequence as in clause 7.1.3.2.1.3.2.

7.1.3.2.3.3.3	Specific message contents

Same specific message contents as in clause 7.1.3.2.1.3.3 except integrity protection algorithm ‘nia3 (ZUC)’ and ‘eia3 (ZUC)’.

##### 7.1.3.2.4

##### 7.1.3.2.5

##### 7.1.3.2.6	Integrity protection / Correct functionality of UP integrity protection / multiple DRBs

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when {  Functionality of integrity algorithms is taken into use on the MCG DRB }

then { UE performs correct integrity protection function in PDCP entities associated with MCG DRB }

}

(2)

with { UE in RRC_CONNECTED state }

ensure that {

when {  Functionality of integrity algorithms is taken into use on the SCG DRB }

then { UE performs correct integrity protection function in PDCP entities associated with SCG DRB }

}

7.1.3.2.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 33.401, clauses 7.3.3 and 7.3.4, TS 36.300, clause 14.1 and TS 36.331, clause.5.3.1.2. Unless otherwise stated these are Rel-17 requirements.

[TS 33.401, clause 7.3.3]

NOTE a:	Only EN-DC capable UEs and EN-DC capable eNBs support UP integrity protection. Therefore, the eNB can only activate UP integrity protection with a UE that is EN-DC capable and supports user plane integrity protection with EPS.

If the UE indicates that it supports user plane integrity protection with EPS in EIA7 in the EPS security capability, the MME shall provide UP integrity protection policy for each E-RAB to the eNB during the Attach/Dedicated bearer activation/Dedicated bearer modification procedure as specified in TS 23.401 [2]. The MME receives UP integrity protection policy from SMF+PGW-C via SGW.

NOTE 1:	The SMF+PGW-C can be locally configured with UP integrity protection and confidentiality policy. However, the SMF+PGW-C only sends UP integrity protection policy to the upgraded SGW. The SMF+PGW-C, SGW and MME can use GTP-C signalling compatibility concepts to jugde whether to send UP integrity protection policy to the peer.

The UP integrity protection policy shall indicate whether UP integrity protection shall be activated or not for all DRBs belonging to that E-RAB.

The eNB shall be locally configured with UP integrity protection policy. If the eNB receives UP integrity protection policy from the MME, the eNB shall use the received UP integrity protection policy, otherwise, the eNB shall use the locally configured UP integrity protection policy if EIA7 in the EPS security capability indicates that the UE supports user plane integrity protection with EPC.

The locally configured UP integrity protection policy on eNB should be set as "preferred".

The eNB shall activate UP integrity protection per each DRB, according to the UP integrity protection policy, using RRC signalling as defined in clause 7.3.4. If the UP integrity protection policy indicates "Required", the eNB shall activate UP integrity protection. If the eNB cannot activate UP integrity protection, and when the UP integrity protection policy is "Required", the eNB shall reject establishment of UP resources for the E-RAB and indicate reject-cause to the MME. If the UP integrity protection policy is " Not needed ", the eNB shall not activate UP integrity protection.

…

[TS 33.401, clause 7.3.4]

AS UP integrity protection activation shall be done as part of the DRB addition procedure using RRC Connection Reconfiguration procedure as described in this clause, see Figure 7.3.4 -1.

As defined in Clause 7.3.3, the MME may send the UP integrity protection policy to the eNB. If the MME does not send the UP integrity protection policy, the eNB may use locally configured UP integrity protection policy.

Figure 7.3.4-1: User plane (UP) integrity protection activation mechanism

1a.	This RRC Connection Reconfiguration procedure which is used to add DRBs shall be performed only after RRC security and UP ciphering have been activated as part of the AS security mode command procedure defined in Clause 7.2.4.5 and the UE indicates that it supports use of user plane integrity protection with EPC.

1b.	The eNB shall send the RRC Connection Reconfiguration message to the UE for UP security activation containing indication for the activation of UP integrity protection for each DRB according to the security policy.

The eNB shall select the NR integrity algorithm and indicate it in the RRC Connection Reconfiguration procedure to the UE. The selected NR integrity algorithm  corresponds to the EPS integrity algorithm which the eNB selected and indicated to the UE in the AS Security Mode Command procedure.

1c.	If UP integrity protection is activated for DRBs as indicated in the RRC Connection Reconfiguration message, and if the eNB does not have KUPint, the eNB shall generate KUPint and UP integrity protection for such DRBs shall start at the eNB.

2a.	UE shall verify the RRC Connection Reconfiguration message. If successful, if UP integrity protection is activated for DRBs as indicated in the RRC Connection Reconfiguration message, and if the UE does not have KUPint, the UE shall generate KUPint and UP integrity protection for such DRBs shall start at the UE.

2b.	If the UE successfully verifies integrity of the RRC Connection Reconfiguration message, the UE shall send the RRC Connection Reconfiguration Complete message to the eNB.

When the UE receives the RRC Connection Reconfiguration message then the UE shall use the EPS algorithm which corresponds to the NR algorithm indicated in the RRC Connection Reconfiguration message for UP integrity protection.

If UP integrity protection is not activated for DRBs, the eNB and the UE shall not integrity protect the traffic of such DRB and shall not put MAC-I into PDCP packet.

[TS 36.300, clause 14.1]

The following principles apply to E-UTRAN security:

…

-	Separate AS and NAS level security mode command procedures are used. AS level security mode command procedure configures AS security (RRC and user plane) and NAS level security mode command procedure configures NAS security. Both integrity protection and ciphering for RRC are activated within the same AS SMC procedure. User plane ciphering is activated at the same time as RRC ciphering. An EN-DC capable UE supporting user plane integrity protection (see TS 24.301 [20]) when connected to E-UTRA/EPC (as specified in TS 33.401 [22]) shall support integrity protection for all DRBs (MN and SN terminated) at any data rate, up to and including the highest data rate supported by the UE for both UL and DL. When supported, user plane integrity protection with NR PDCP can be activated (on a per radio bearer basis) upon DRB addition.

…

[TS 36.331, clause.5.3.1.2]

…

The AS applies different security keys: one for the integrity protection of RRC signalling (KRRCint), one for the ciphering of RRC signalling (KRRCenc) and one for the ciphering of user data (KUPenc). For the UE capable of user plane integrity protection when it is connected to E-UTRA/EPC (TS 33.401 [32]), the AS applies a security key for integrity protection of user data (KUPint) for the DRBs that are configured to apply integrity protection of user data. All AS keys are derived from the KeNB key. The KeNB is based on the KASME key for E-UTRA/EPC, or KAMF for E-UTRA/5GC, which is handled by upper layers.

…

7.1.3.2.6.3	Test description

7.1.3.2.6.3.1	Pre-test conditions

-	Same Pre-test conditions as in clause 7.1.3.0 with Connectivity set to EN-DC without message condition UE TEST LOOP MODE A, then RRCConnectionReconfiguration message including MobilityControlInfo IE is transmitted on E-UTRA Cell 1 to reconfigure SRB1, SRB2 and MCG DRB from E-UTRA PDCP to NR PDCP and Test Loop Function (On) with UE test loop mode A (message condition UE TEST LOOP MODE A to return one UL PDCP SDU per DL PDCP SDU) according to TS 38.508-1 [4]

7.1.3.2.6.3.2	Test procedure sequence

Table 7.1.3.2.6.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits PDCP PDU on MCG DRB integrity protected. | <-- | PDCP PDU | - | - |
| 2 | Check: Does the UE transmit looped back PDCP PDU integrity protected on MCG DRB? | --> | PDCP PDU | 1 | P |
| 3 | SS transmits PDCP PDU on SCG DRB integrity protected. | <-- | PDCP PDU | - | - |
| 4 | Check: Does the UE transmit looped back PDCP PDU integrity protected on SCG DRB? | --> | PDCP PDU | 2 | P |

7.1.3.2.6.3.3	Specific message contents

Table 7.1.3.2.6.3.3-1: ATTACH REQUEST (Preamble)

| Derivation Path: TS 36.508 [7] Table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE network capability |  |  |  |  |
| All octets with the exception of octet 4, bit 1 |  | Any allowed value |  |  |
| EPS-UPIP supported (octet 4, bit 1) |  | 1 | EPS-UPIP supported |  |

Table 7.1.3.2.6.3.3-2: SecurityModeCommand message (Preamble)

| Derivation Path: TS 36.508 [7] Table 4.6.1-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SecurityModeCommand ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| securityModeCommand-r8 SEQUENCE { |  |  |  |  |
| securityConfigSMC SEQUENCE { |  |  |  |  |
| securityAlgorithmConfig SEQUENCE { |  |  |  |  |
| cipheringAlgorithm |  | Set according to PIXIT parameter for default ciphering algorithm |  |  |
| integrityProtAlgorithm |  | Set according to PIXIT parameter for default integrity protection algorithm | null algorithm is not allowed |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.3.2.6.3.3-3: RRCConnectionReconfiguration (Preamble)

| Derivation Path: TS 36.508[7] Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |  |
| mobilityControlInfo |  | MobilityControlInfo-HO-SameCell | As per Table 7.1.3.2.6.3.3-4 |  |
| radioResourceConfigDedicated |  | RadioResourceConfigDedicated-DRB-Rel-Add | As per Table 7.1.3.2.6.3.3-5 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nr-Config-r15 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| nr-SecondaryCellGroupConfig-r15 | OCTET STRING including the RRCReconfiguration message according to TS 38.508-1 [4], table 4.6.1-13 with condition EN-DC_HO. |  |  |
| } |  |  |  |
| } |  |  |  |
| sk-Counter-r15 | Increment the value by 1 from the previous value |  |  |
| nr-RadioBearerConfig1-r15 | RadioBearerConfig-MCG-SCG-DRB-NR-PDCP | Table 7.1.3.2.6.3.3-6 |  |
| nr-RadioBearerConfig2-r15 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.3.2.6.3.3-4: MobilityControlInfo-HO-SameCell (Table7.1.3.2.6.3.3-3)

| Derivation Path: TS 36.508 [7], Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo-HO ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of E-UTRA Cell 1 |  |  |
| carrierFreq | Not present |  |  |
| } |  |  |  |

Table 7.1.3.2.6.3.3-5: RadioResourceConfigDedicated-DRB-Rel-Add (Table 7.1.3.2.6.3.3-3)

| Derivation Path: TS 36.508 [7], Table 4.6.3-19AAAAAD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment |  | Condition |
| RadioResourceConfigDedicated-SRB2-DRB ::= SEQUENCE { |  |  |  |  |  |  |
| srb-ToAddModList | Not present |  |  |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |  |  |  |
| DRB-ToAddMod[1] | DRB-ToAddMod-DEFAULT (8) using condition AM except pdcp-Config not included |  | entry 1 See TS 36.508 subclause 4.8.2 |  |  |  |
| } |  |  |  |  |  |  |
| drb-ToReleaseList SEQUENCE (SIZE (1..maxDRB)) OF DRB-Identity { | 1 entry |  |  |  |  |  |
| DRB-Identity[1] | 8 |  | entry 1 Same as the DRB Identity associated with the default EPS bearer |  |  |  |
| } |  |  |  |  |  |  |
| physicalConfigDedicated | PhysicalConfigDedicated-DEFAULT with condition RBC-HO |  |  |  |  |  |
| } |  |  |  |  |  |  |

Table 7.1.3.2.6.3.3-6: RadioBearerConfig-MCG-SCG-DRB-NR-PDCP (Table 7.1.3.2.6.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-132, condition MCG_NR_PDCP, SRB_NR_PDCP AND Re-establish_PDCP |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 2 entries |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| eps-BearerIdentity | Same as the default EPS bearer Identity | MCG DRB |  |
| } |  |  |  |
| drb-Identity | Same as the DRB identity associated with the default EPS bearer |  |  |
| reestablishPDCP | Not present |  |  |
| recoverPDCP | Not present |  |  |
| pdcp-Config | PDCP-Config | Table 7.1.3.2.6.3.3-7 |  |
| } |  |  |  |
| DRB-ToAddMod[2] SEQUENCE { |  | entry 2 |  |
| cnAssociation CHOICE { |  |  |  |
| eps-BearerIdentity | 6 |  |  |
| } |  |  |  |
| drb-Identity | DRB-Identity using condition DRB2 |  |  |
| reestablishPDCP | true |  |  |
| recoverPDCP | Not present |  |  |
| pdcp-Config | PDCP-Config | Table 7.1.3.2.6.3.3-7 |  |
| } |  |  |  |
| } |  |  |  |
| securityConfig SEQUENCE { |  |  |  |
| securityAlgorithmConfig SEQUENCE { |  |  |  |
| cipheringAlgorithm | Same as the ciphering algorithm configured Table 7.1.3.2.6.3.3-2 |  |  |
| integrityProtAlgorithm | Same as the integrity algorithm configured Table 7.1.3.2.6.3.3-2 |  |  |
| } |  |  |  |
| keyToUse | secondary |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.2.6.3.3-7: PDCP-Config (Table 7.1.3.2.6.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-99 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-Config ::= SEQUENCE { |  |  |  |
| drb SEQUENCE { |  |  |  |
| integrityProtection | enabled |  |  |
| } |  |  |  |
| } |  |  |  |

#### 7.1.3.3	PDCP Ciphering and deciphering

##### 7.1.3.3.1	Ciphering and deciphering / Correct functionality of encryption algorithm SNOW3G / SRB / DRB

7.1.3.3.1.1	Test Purpose (TP)

(1)

(1)

with { UE in RRC_CONNECTED state and SRB is configured with NR-PDCP }

ensure that {

when { Functionality of encryption algorithms with SNOW3G is taken into use on SRB }

then { UE performs correct ciphering/deciphering function in NR-PDCP entity associated with SRB }

}

(2)

with { UE in RRC_CONNECTED state }

ensure that {

when { Functionality of encryption algorithms with SNOW3G is taken into use on DRB }

then {UE performs correct ciphering/deciphering function in NR-PDCP entity associated with DRB }

}

7.1.3.3.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.323 clause 5.8, TS 33.501 clauses 5.6.1, D.2.1.1 and TS 36.331 clause 6.3.2. Unless otherwise stated these are Rel-15 requirements.

[TS 38.323, clause 5.8]

The ciphering function includes both ciphering and deciphering and is performed in PDCP, if configured. The data unit that is ciphered is the data part of the PDCP Data PDU (see subclause 6.3.3) except the SDAP header if included in the PDCP SDU, and the MAC-I (see subclause 6.3.4). The ciphering is not applicable to PDCP Control PDUs.

The ciphering algorithm and key to be used by the PDCP entity are configured by upper layers TS 38.331 [3] and the ciphering method shall be applied as specified in TS 33.501 [6].

The ciphering function is activated by upper layers TS 38.331 [3]. When security is activated, the ciphering function shall be applied to all PDCP Data PDUs indicated by upper layers TS 38.331 [3] for the downlink and the uplink, respectively.

For downlink and uplink ciphering and deciphering, the parameters that are required by PDCP for ciphering are defined in TS 33.501 [6] and are input to the ciphering algorithm. The required inputs to the ciphering function include the COUNT value, and DIRECTION (direction of the transmission: set as specified in TS 33.501 [6]).The parameters required by PDCP which are provided by upper layers TS 38.331 [3] are listed below:

-	BEARER (defined as the radio bearer identifier in TS 33.501 [6]. It will use the value RB identity –1 as in TS 38.331 [3]);

-	KEY (the ciphering keys for the control plane and for the user plane are KRRCenc and KUPenc, respectively).

[TS 33.501, clause 5.6.1]

All Identifiers and names specified in this subclause are for5G.

Each Encryption Algorithm used for 5G will be assigned a 4-bit identifier. The following values for ciphering algorithms are defined:

"00002"         NEA0			Null ciphering algorithm;

"00012"         128-NEA1		128-bit SNOW 3G based algorithm;

"00102"         128-NEA2		128-bit AES based algorithm; and

"00112"         128-NEA3		128-bit ZUC based algorithm.

128-NEA1 is based on SNOW 3G (see TS35.215 [14]).

128-NEA2 is based on 128-bit AES [15] in CTR mode [16].

128-NEA3 is based on 128-bit ZUC (sseTS35.221 [18]).

Full details of the algorithms are specified in Annex D.

[TS 33.501, clause D.2.1.1]

The input parameters to the ciphering algorithm are a 128-bit cipher key named KEY, a 32-bit COUNT, a 5-bit bearer identity BEARER, the 1-bit direction of the transmission i.e. DIRECTION, and the length of the keystream required i.e. LENGTH. The DIRECTION bit shall be 0 for uplink and 1 for downlink.

Figure D.2.1.1-1 illustrates the use of the ciphering algorithm NEA to encrypt plaintext by applying a keystream using a bit per bit binary addition of the plaintext and the keystream. The plaintext may be recovered by generating the same keystream using the same input parameters and applying a bit per bit binary addition with the ciphertext.

Figure D.2.1.1-1: Ciphering of data

Based on the input parameters the algorithm generates the output keystream block KEYSTREAM which is used to encrypt the input plaintext block PLAINTEXT to produce the output ciphertext block CIPHERTEXT.

The input parameter LENGTH shall affect only the length of the KEYSTREAM BLOCK, not the actual bits in it.

[TS 36.331, clause 6.3.3]

The IE SecurityAlgorithmConfig is used to configure AS integrity protection algorithm (SRBs) and AS ciphering algorithm (SRBs and DRBs).

…

SecurityAlgorithmConfig field descriptions

cipheringAlgorithm Indicates the ciphering algorithm to be used for SRBs and DRBs, as specified in TS 33.501 [11]. The algorithms nea0-nea3 are identical to the LTE algorithms eea0-3. For EN-DC, the algorithms configured for bearers using KeNB shall be the same as for all bearers using KeNB.

integrityProtAlgorithm For EN-DC, this IE indicates the integrity protection algorithm to be used for SRBs, as specified in TS 33.501 [11]. The algorithms nia0-nia3 is identical to the LTE algorithms eia0-3. For EN-DC, the algorithms configured for SRBs using KeNB shall be the same as for all SRBs using KeNB.

7.1.3.3.1.3	Test description

7.1.3.3.1.3.1	Pre-test conditions

-	Same Pre-test conditions as in clause 7.1.3.0 with the exception that ciphering algorithm ‘nea1 (SNOW3G)’ is configured.

-	For EN_DC or NGEN_DC, same Pre-test conditions as in clause 7.1.3.0 with the exception that ciphering algorithms ‘eea1 (SNOW3G)’ and ‘nea1 (SNOW3G)’ are configured and without message condition UE TEST LOOP MODE A, then RRCConnectionReconfiguration message including MobilityControlInfo IE is transmitted on E-UTRA Cell 1 to reconfigure SRB1, SRB2 and MCG DRB from E-UTRA PDCP to NR PDCP and Test Loop Function (On) with UE test loop mode A (message condition UE TEST LOOP MODE A to return one UL PDCP SDU per DL PDCP SDU) according to TS 38.508-1 [4].

7.1.3.3.1.3.2	Test procedure sequence

Table 7.1.3.3.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| - | Exception steps 1a1 to 1b2 depends on UE configuration. | - | - | - | - |
| 1a1 | IF Connectivity is EN-DC or NGEN-DC, the SS sends EUTRA RRC UECapabilityEnquiry including RAT-Type eutra message to the UE. | <-- | RRC: UECapabilityEnquiry | - | - |
| 1a2 | Check: Does the UE send a EUTRA RRC UECapabilityInformation message? | --> | RRC: UECapabilityInformation | 1 | P |
| 1b1 | ELSE the SS sends NR RRC UECapabilityEnquiry message to the UE. | <-- | NR RRC: UECapabilityEnquiry | - | - |
| 1b2 | Check: Does the UE send a NR RRC UECapabilityInformation message? | --> | NR RRC: UECapabilityInformation | 1 | P |
| - | EXCEPTION: steps 2a1-2a2 depends on UE configuration, executed if SCG DRB is configured | - | - | - | - |
| 2a1 | IF Connectivity is EN-DC or NGEN-DC, SS transmits PDCP PDU on SCG DRB ciphered. | <-- | PDCP PDU | - | - |
| 2a2 | Check: Does the UE transmit looped back PDCP PDU ciphered on SCG DRB? | --> | PDCP PDU | 2 | P |
| 3 | SS transmits PDCP PDU on MCG DRB ciphered. | <-- | PDCP PDU | - | - |
| 4 | Check: Does the UE transmit looped back PDCP PDU ciphered on MCG DRB? | --> | PDCP PDU | 2 | P |

7.1.3.3.1.3.3	Specific message contents

Table 7.1.3.3.1.3.3-1: RRCConnectionReconfiguration (Preamble for EN-DC or NGEN-DC)

| Derivation Path: 36.508[47] Table 4.6.1-8 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  |  | Value/remark |  | Comment |  | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |  |  |  |  |
| mobilityControlInfo |  |  | MobilityControlInfo-HO-SameCell |  | As per Table 7.1.3.3.1.3.3-2 |  |  |
| radioResourceConfigDedicated |  |  | RadioResourceConfigDedicated-DRB-Rel-Add |  | As per Table 7.1.3.3.1.3.3-3 |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |
| nr-Config-r15 CHOICE { |  |  |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |  |  |
| nr-SecondaryCellGroupConfig-r15 | OCTET STRING including the RRCReconfiguration message according to TS 38.508-1 [4], table 4.6.1-13 with condition EN-DC_HO. |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| sk-Counter-r15 | Increment the value by 1 from the previous value |  |  |  |  |  |
| nr-RadioBearerConfig1-r15 | OCTET STRING including RadioBearerConfig according to TS 38.508-1 [4], table 4.6.3-132 with condition EN-DC_DRB and Re-establish_PDCP |  |  |  |  |  |
| nr-RadioBearerConfig2-r15 | OCTET STRING including RadioBearerConfig according to TS 38.508-1 [4], table 4.6.3-132 with conditions SRB_NR_PDCP and MCG_NR_PDCP |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 7.1.3.3.1.3.3-2: MobilityControlInfo-HO-SameCell (Table 7.1.3.3.1.3.3-1)

| Derivation Path: 36.508 [7], Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo-HO ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of E-UTRA Cell 1 |  |  |
| carrierFreq | Not present |  |  |
| } |  |  |  |

Table 7.1.3.3.1.3.3-3: RadioResourceConfigDedicated-DRB-Rel-Add (Table 7.1.3.3.1.3.3-1)

| Derivation Path: 36.508 [7], Table 4.6.3-19AAAAAD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment |  | Condition |
| RadioResourceConfigDedicated-SRB2-DRB ::= SEQUENCE { |  |  |  |  |  |  |
| srb-ToAddModList | Not present |  |  |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |  |  |  |
| drb-ToAddMod[1] | DRB-ToAddMod-DEFAULT (8) using condition AM except pdcp-Config not included |  | entry 1 See TS 36.508 subclause 4.8.2 |  |  |  |
| } |  |  |  |  |  |  |
| drb-ToReleaseList SEQUENCE (SIZE (1..maxDRB)) OF DRB-Identity { | 1 entry |  |  |  |  |  |
| DRB-Identity[1] | 8 |  | entry 1 Same as the DRB Identity associated with the default EPS bearer |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |

##### 7.1.3.3.2	Ciphering and deciphering / Correct functionality of encryption algorithm AES / SRB / DRB

7.1.3.3.2.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and SRB is configured with NR-PDCP }

ensure that {

when { Functionality of encryption algorithms with AES is taken into use on SRB }

then { UE performs correct ciphering/deciphering function in NR-PDCP entity associated with SRB }

}

(2)

with { UE in RRC_CONNECTED state }

ensure that {

when { Functionality of encryption algorithms with AES is taken into use on DRB }

then {UE performs correct ciphering/deciphering function in NR-PDCP entity associated with DRB }

}

7.1.3.3.2.2	Conformance requirements

Same conformance requirement as in clause 7.1.3.3.1.2.

7.1.3.3.2.3	Test description

7.1.3.3.2.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.3.3.1.3.1 with the exception that ciphering algorithm ‘nea2 (AES)’ and ‘eea2 (AES)’ is configured.

7.1.3.3.2.3.2	Test procedure sequence

Same Test procedure sequence as in clause 7.1.3.3.1.3.2

7.1.3.3.2.3.3	Specific message contents

None

##### 7.1.3.3.3	Ciphering and deciphering / Correct functionality of encryption algorithm ZUC / SRB / DRB

7.1.3.3.3.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and SRB is configured with NR-PDCP}

ensure that {

when { Functionality of encryption algorithms with ZUC is taken into use on SRB }

then { UE performs correct ciphering/deciphering function in NR-PDCP entity associated with SRB }

}

(2)

with { UE in RRC_CONNECTED state and DRB is configured with NR-PDCP}

ensure that {

when { Functionality of encryption algorithms with ZUC is taken into use on DRB }

then { UE performs correct ciphering/deciphering function in NR-PDCP entity associated with DRB }

}

7.1.3.3.3.2	Conformance requirements

Same conformance requirement as in clause 7.1.3.3.1.2.

7.1.3.3.3.3	Test description

7.1.3.3.3.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.3.3.1.3.1 with the exception that ciphering algorithm ‘nea3 (ZUC)’ and ‘eea3 (ZUC)’ is configured.

7.1.3.3.3.3.2	Test procedure sequence

Same Test procedure sequence as in clause 7.1.3.3.1.3.2.

7.1.3.3.3.3.3	Specific message contents

None

#### 7.1.3.4	PDCP Handover

##### 7.1.3.4.1	PDCP handover / Lossless handover / PDCP sequence number maintenance / PDCP status report to convey the information on missing or acknowledged PDCP SDUs at handover / In-order delivery and duplicate elimination in the downlink

7.1.3.4.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with default RB using RLC-AM }

ensure that {

when { UE is requested to make a lossless handover by SS }

then { UE creates a PDCP status report to SS }

}

(2)

with { UE in RRC_CONNECTED state with default RB using RLC-AM }

ensure that {

when { UE is requested to make a lossless handover by SS }

then { UE retransmits the unacknowledged data }

}

(3)

with { UE in RRC_CONNECTED state with default RB using RLC-AM }

ensure that {

when { UE is requested to make a lossless handover by SS }

then { UE achieves in-order delivery and discards a PDCP PDU already received in the downlink }

}

7.1.3.4.1.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.323, clauses 5.1.2, 5.2.2.1, 5.3, 5.4.1, 5.4.2 and 7.1. Unless otherwise stated these are Rel-15 requirements.

[TS 38.323, clause 5.1.2]

When upper layers request a PDCP entity re-establishment, the UE shall additionally perform once the procedures described in this section. After performing the procedures in this section, the UE shall follow the procedures in subclause 5.2.

When upper layers request a PDCP entity re-establishment, the transmitting PDCP entity shall:

-	for UM DRBs and AM DRBs, reset the header compression protocol for uplink and start with an IR state in U-mode (as defined in RFC 3095 [8] and RFC 4815 [9]) if drb-ContinueROHC is not configured in TS 38.331 [3];

-	for UM DRBs and SRBs, set TX_NEXT to the initial value;

-	for SRBs, discard all stored PDCP SDUs and PDCP PDUs;

-	apply the ciphering algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	apply the integrity protection algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	for UM DRBs, for each PDCP SDU already associated with a PDCP SN but for which a corresponding PDU has not previously been submitted to lower layers:

-	consider the PDCP SDUs as received from upper layer;

-	perform transmission of the PDCP SDUs in ascending order of the COUNT value associated to the PDCP SDU prior to the PDCP re-establishment without restarting the discardTimer, as specified in subclause 5.2.1;

-	for AM DRBs, from the first PDCP SDU for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by lower layers, perform retransmission or transmission of all the PDCP SDUs already associated with PDCP SNs in ascending order of the COUNT values associated to the PDCP SDU prior to the PDCP entity re-establishment as specified below:

-	perform header compression of the PDCP SDU as specified in the subclause 5.7.4;

-	perform integrity protection and ciphering of the PDCP SDU using the COUNT value associated with this PDCP SDU as specified in the subclause 5.9 and 5.8;

-	submit the resulting PDCP Data PDU to lower layer, as specified in subclause 5.2.1.

When upper layers request a PDCP entity re-establishment, the receiving PDCP entity shall:

-	process the PDCP Data PDUs that are received from lower layers due to the re-establishment of the lower layers, as specified in the subclause 5.2.2.1;

-	for SRBs, discard all stored PDCP SDUs and PDCP PDUs;

-	for SRBs and UM DRBs, if t-Reordering is running:

-	stop and reset t-Reordering;

-	for UM DRBs, deliver all stored PDCP SDUs to the upper layers in ascending order of associated COUNT values after performing header decompression;

-	for AM DRBs, perform header decompression for all stored PDCP SDUs if drb-ContinueROHC is not configured in TS 38.331 [3];

-	for UM DRBs and AM DRBs, reset the header compression protocol for downlink and start with NC state in U-mode (as defined in RFC 3095 [8] and RFC 4815 [9]) if drb-ContinueROHC is not configured in TS 38.331 [3];

-	for UM DRBs and SRBs, set RX_NEXT and RX_DELIV to the initial value;

-	apply the ciphering algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	apply the integrity protection algorithm and key provided by upper layers during the PDCP entity re-establishment procedure.

[TS 38.323, clause 5.2.2.1]

In this section, following definitions are used:

-	HFN(State Variable): the HFN part (i.e. the number of most significant bits equal to HFN length) of the State Variable;

-	SN(State Variable): the SN part (i.e. the number of least significant bits equal to PDCP SN length) of the State Variable;

-	RCVD_SN: the PDCP SN of the received PDCP Data PDU, included in the PDU header;

-	RCVD_HFN: the HFN of the received PDCP Data PDU, calculated by the receiving PDCP entity;

-	RCVD_COUNT: the COUNT of the received PDCP Data PDU = [RCVD_HFN, RCVD_SN].

At reception of a PDCP Data PDU from lower layers, the receiving PDCP entity shall determine the COUNT value of the received PDCP Data PDU, i.e. RCVD_COUNT, as follows:

-	if RCVD_SN < SN(RX_DELIV) – Window_Size:

-	RCVD_HFN = HFN(RX_DELIV) + 1.

-	else if RCVD_SN >= SN(RX_DELIV) + Window_Size:

-	RCVD_HFN = HFN(RX_DELIV) – 1.

-	else:

-	RCVD_HFN = HFN(RX_DELIV);

-	RCVD_COUNT = [RCVD_HFN, RCVD_SN].

After determining the COUNT value of the received PDCP Data PDU = RCVD_COUNT, the receiving PDCP entity shall:

-	perform deciphering and integrity verification of the PDCP Data PDU using COUNT = RCVD_COUNT;

-	if integrity verification fails:

-	indicate the integrity verification failure to upper layer;

-	discard the PDCP Data PDU;

-	if RCVD_COUNT < RX_DELIV; or

-	if the PDCP Data PDU with COUNT = RCVD_COUNT has been received before:

-	discard the PDCP Data PDU;

If the received PDCP Data PDU with COUNT value = RCVD_COUNT is not discarded above, the receiving PDCP entity shall:

-	store the resulting PDCP SDU in the reception buffer;

-	if RCVD_COUNT >= RX_NEXT:

-	update RX_NEXT to RCVD_COUNT + 1.

-	if outOfOrderDelivery is configured:

-	deliver the resulting PDCP SDU to upper layers.

-	if RCVD_COUNT = RX_DELIV:

-	deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before;

-	all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from COUNT = RX_DELIV;

-	update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers, with COUNT value > RX_DELIV;

-	if t-Reordering is running, and if RX_DELIV >= RX_REORD:

-	stop and reset t-Reordering.

-	if t-Reordering is not running (includes the case when t-Reordering is stopped due to actions above), and RX_DELIV < RX_NEXT:

-	update RX_REORD to RX_NEXT;

-	start t-Reordering.

[TS 38.323, clause 5.3]

When the discardTimer expires for a PDCP SDU, or the successful delivery of a PDCP SDU is confirmed by PDCP status report, the transmitting PDCP entity shall discard the PDCP SDU along with the corresponding PDCP Data PDU. If the corresponding PDCP Data PDU has already been submitted to lower layers, the discard is indicated to lower layers.

For SRBs, when upper layers request a PDCP SDU discard, the PDCP entity shall discard all stored PDCP SDUs and PDCP PDUs.

NOTE:	Discarding a PDCP SDU already associated with a PDCP SN causes a SN gap in the transmitted PDCP Data PDUs, which increases PDCP reordering delay in the receiving PDCP entity. It is up to UE implementation how to minimize SN gap after SDU discard.

[TS 38.323, clause 5.4.1]

For AM DRBs configured by upper layers to send a PDCP status report in the uplink (statusReportRequired in TS 38.331 [3]), the receiving PDCP entity shall trigger a PDCP status report when:

-	upper layer requests a PDCP entity re-establishment;

-	upper layer requests a PDCP data recovery.

If a PDCP status report is triggered, the receiving PDCP entity shall:

-	compile a PDCP status report as indicated below by:

-	setting the FMC field to RX_DELIV;

-	if RX_DELIV < RX_NEXT:

-	allocating a Bitmap field of length in bits equal to the number of COUNTs from and not including the first missing PDCP SDU up to and including the last out-of-sequence PDCP SDUs, rounded up to the next multiple of 8, or up to and including a PDCP SDU for which the resulting PDCP Control PDU size is equal to 9000 bytes, whichever comes first;

-	setting in the bitmap field as '0' for all PDCP SDUs that have not been received, and optionally PDCP SDUs for which decompression have failed;

-	setting in the bitmap field as '1' for all PDCP SDUs that have been received;

-	submit the PDCP status report to lower layers as the first PDCP PDU for transmission via the transmitting PDCP entity as specified in subclause 5.2.1..

[TS 38.323, clause 5.4.2]

For AM DRBs, when a PDCP status report is received in the downlink, the transmitting PDCP entity shall:

-	consider for each PDCP SDU, if any, with the bit in the bitmap set to '1', or with the associated COUNT value less than the value of FMC field as successfully delivered, and discard the PDCP SDU as specified in the subclause 5.3.

[TS 38.323, clause 7.1]

This sub clause describes the state variables used in PDCP entities in order to specify the PDCP protocol. The state variables defined in this subclause are normative.

All state variables are non-negative integers, and take values from 0 to [232 – 1].

PDCP Data PDUs are numbered integer sequence numbers (SN) cycling through the field: 0 to [2[pdcp-SN-Size] – 1].

The transmitting PDCP entity shall maintain the following state variables:

a)	TX_NEXT

This state variable indicates the COUNT value of the next PDCP SDU to be transmitted. The initial value is 0.

The receiving PDCP entity shall maintain the following state variables:

a)	RX_NEXT

This state variable indicates the COUNT value of the next PDCP SDU expected to be received. The initial value is 0.

b)	RX_DELIV

This state variable indicates the COUNT value of the first PDCP SDU not delivered to the upper layers, but still waited for. The initial value is 0.

c)	RX_REORD

This state variable indicates the COUNT value following the COUNT value associated with the PDCP Data PDU which triggered t-Reordering.

7.1.3.4.1.3	Test description

7.1.3.4.1.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.3.0 except the following:

-	2 NR cells (NR Cell 1 and NR Cell 2) are configured with DRBs in RLC AM mode.

-	The cell power levels are configured as per the Table 7.1.3.4.1.3.1-1/2.

-	DRB of NR Cell 1 is configured according to Table 7.1.3.4.1.3.1-3.

Table 7.1.3.4.1.3.1-1: Time instances of cell power level in FR1

|  | Parameter | Unit | EUTRA Cell 1 | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/SCS | -85 | - | - |  |
|  | SS/PBCH SSS EPRE | dBm/SCS | - | -88 | Off |  |
| T1 | Cell-specific RS EPRE | dBm/SCS | -85 | - | - |  |
|  | SS/PBCH SSS EPRE | dBm/SCS | - | -88 | -82 |  |
| T2 | Cell-specific RS EPRE | dBm/SCS | -85 | - | - |  |
|  | SS/PBCH SSS EPRE | dBm/SCS | - | -82 | -88 |  |

Table 7.1.3.4.1.3.1-2: Time instances of cell power level in FR2

|  | Parameter | Unit | EUTRA Cell 1 | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/SCS | -96 | - | - |  |
|  | SS/PBCH SSS EPRE | dBm/SCS | - | -91 | Off |  |
| T1 | Cell-specific RS EPRE | dBm/SCS | -96 | - | - |  |
|  | SS/PBCH SSS EPRE | dBm/SCS | - | -91 | -82 |  |
| T2 | Cell-specific RS EPRE | dBm/SCS | -96 | - | - |  |
|  | SS/PBCH SSS EPRE | dBm/SCS | - | -82 | -91 |  |

Table 7.1.3.4.1.3.1-3: RLC parameters

| t-PollRetransmit | ms150 |
| --- | --- |

Table 7.1.3.4.1.3.1-4: MAC-CellGroupConfig

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-68 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MAC-CellGroupConfig ::= SEQUENCE { |  |  |  |
| bsr-Config SEQUENCE { |  |  |  |
| retxBSR-Timer | sf80 |  |  |
| } |  |  |  |
| } |  |  |  |

7.1.3.4.1.3.2	Test procedure sequence

Table 7.1.3.4.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS creates 5 PDCP Data PDUs and the TX_NEXT is set to "0". | - | - | - | - |
| - | EXCEPTION: Step 2 and 3 shall be repeated for k=0 to 1(increment=1). | - | - | - | - |
| 2 | The SS sends the PDCP Data PDU#k via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = k on NR Cell 1. After having sent a PDU, the SS set TX_NEXT = k+1. | <-- | PDCP PDU DATA #k | - | - |
| 3 | The UE sends the PDCP Data PDU#k via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = k on NR Cell 1. Data is previously received data from PDU #k. (Note 1) | --> | PDCP PDU DATA #k | - | - |
| 3A | The SS changes NR Cell 2 parameters according to the row "T1" in table 7.1.3.4.1.3.1-1(FR1) / 7.1.3.4.1.3.1-2(FR2). | - | - | - | - |
| - | EXCEPTION: Step 4 to 6 shall be repeated for m=2 to 4 (increment=1). | - | - | - | - |
| 4 | The SS is configured on NR Cell 1 not to send RLC acknowledgement (RLC ACK) to the next received RLC SDU to the UE. | - | - | - | - |
| 5 | The SS sends the PDCP Data PDU #m via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN =m. After having sent a PDU, the SS set TX_NEXT = m+1. (Note 6) | <-- | PDCP PDU DATA #m | - | - |
| 6 | The UE sends the PDCP Data PDU#m via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = m. Data is previously received data from PDU #m. (Note 2) | --> | PDCP PDU DATA #m | - | - |
| 6A | Configure SS not to allocate UL grant to the UE in NR Cell 1 | - | - | - | - |
| 7 | Void | - | - | - | - |
| 8 | The SS transmits NR RRCReconfiguration message to perform SpCell change from NR Cell 1 to NR Cell 2 (Note 3) | <-- | RRCReconfiguration | - | - |
| 9 | The SS assigns UL grant during the Random Access procedure on NR Cell 2 to allow the UE to send PDCP status report.(Note 5) | - | - | - | - |
| - | EXCEPTION: Steps 10 and 11 can occur in any order. (Note 7) | - | - | - | - |
| 10 | The UE transmits a NR RRCReconfigurationComplete message. (Note 4) | --> | RRCReconfigurationComplete | - | - |
| 11 | Check: Does the UE send PDCP Control PDUs via RLC-AM RB with the following content to the SS: D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 5 on NR Cell 2? | --> | PDCP STATUS REPORT | 1 | P |
| 12 | The SS generates a PDCP status report message and sends it to UE: D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 2 on NR Cell 2. | <-- | PDCP STATUS REPORT | - | - |
| 13 | Configure the SS to allocate Default UL grants to the UE in NR Cell 2. | - | - | - | - |
| 14 | Void | - | - | - | - |
| - | EXCEPTION: Step 15 shall be repeated for m=2 to 4 (increment=1). | - | - | - | - |
| 15 | Check: Does the UE send the PDCP Data PDU #m via RLC-AM RB with the following content to the SS: D/C field = 1 (PDCP Data PDU) and PDCP SN = m on NR Cell 2? Note: Data is previously received data from PDU #m. (Note 8) | --> | PDCP PDU DATA #m | 2 | P |
| 16 | The SS sends the PDCP Data PDU#5 via RLC-AM RB with the following content to the UE: PDCP Data PDU #5 ( D/C field = 1 (PDCP Data PDU) and PDCP SN=5) on NR Cell 2. | <-- | PDCP DATA PDU#5 | - | - |
| 17 | The UE transmits a PDCP Data PDU via RLC-AM RB with the following content back to the SS: D/C field = 1 (PDCP Data PDU) and PDCP SN=5 on NR Cell 2. Note: Data is previously received packet in PDCP Data PDU#5. (Note 1) | --> | PDCP DATA PDU #5 | - | - |
| 18 | TX_NEXT is set to "6". The SS creates a PDCP Data PDU#6 (not transmitted). | - | - | - | - |
| 19 | The TX_NEXT is set to "7". The SS creates a PDCP Data PDU #7. | - | - | - | - |
| 20 | The SS sends PDCP Data PDU#7 via RLC-AM RB with the following content to the UE: PDCP Data PDU#7; D/C field = 1 (PDCP Data PDU) and PDCP SN=7 on NR Cell 2. | <-- | PDCP DATA PDU #7 | - | - |
| 21 | Check: Does the UE transmit a PDCP DATA PDU#7 on NR Cell 2? | --> | PDCP DATA PDU#7 | 3 | F |
| 21A | Configure SS not to allocate UL grant to the UE in NR Cell 1. | - | - | - | - |
| 22 | The SS changes NR Cell 1 and NR Cell 2 parameters according to row "T2" in Table 7.1.3.4.1.3.1-1(FR1) / 7.1.3.4.1.3.1-2(FR2). | - | - | - | - |
| 23 | The SS requests transmits NR RRCReconfiguration message to perform SpCell change from NR Cell 2 to NR Cell 1 with key change. (Note 3) | <-- | RRCReconfiguration | - | - |
| 24 | SS assigns UL grant during the Random Access procedure on NR Cell 1 to allow the UE to send PDCP status report.(Note 5) | - | - | - | - |
| - | EXCEPTION: Steps 25 and 26 can occur in any order. (Note 7) | - | - | - | - |
| 25 | The UE transmits a NR RRCReconfigurationComplete message. (Note 4) | --> | RRCReconfigurationComplete | - | - |
| 26 | The UE sends PDCP Control PDUs via RLC-AM RB with the following content to the SS: D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 6, Bitmap = 0x80 on NR Cell 1. | --> | PDCP STATUS REPORT | - | - |
| 27 | The SS generates a PDCP status report message and sends it to UE: D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 6 on NR Cell 1. | <-- | PDCP STATUS REPORT | - | - |
| 28 | Configure the SS to allocate Default UL grants to the UE in NR Cell 1. | - | - | - | - |
| 28A | The SS sends the PDCP Data PDU#5 via RLC-AM RB with the following content to the UE: PDCP Data PDU #5 ( D/C field = 1 (PDCP Data PDU) and PDCP SN=5) on NR Cell 1. | <-- | PDCP DATA PDU#5 | - | - |
| 28B | Check: Does the UE transmit a PDCP Data PDU via RLC-AM RB with the following content back to the SS: D/C field = 1 (PDCP Data PDU) and PDCP SN=5 on NR Cell 1 within the next 5 seconds? | --> | PDCP DATA PDU #5 | 3 | F |
| 29 | The SS sends the PDCP Data PDU#6 via RLC-AM RB with the following content to the UE: PDCP Data PDU#6 ( D/C field = 1 (PDCP Data PDU) and PDCP SN=6) on NR Cell 1. | <-- | PDCP DATA PDU #6 | - | - |
| 30 | Check: Does the UE transmit a PDCP Data PDU via RLC-AM RB with the following content back to the SS? D/C field = 1 (PDCP Data PDU) and PDCP SN=6 on NR Cell 1. Note: Data is previously received packet in PDCP Data PDU#6 (Note 9) | --> | PDCP DATA PDU #6 | 3 | P |
| 31 | Check: Does the UE transmit PDCP Data PDU via RLC-AM RB with the following content back to the SS? D/C field = 1 (PDCP Data PDU) and PDCP SN=7 on NR Cell 1. Note: Data is previously received packet in PDCP Data PDU#7 (Note 9) | --> | PDCP DATA PDU #7 | 3 | P |
| Note 1: The SS acknowledges the received data. Note 2: SS doesn’t send the RLC ACK for this data. Note 3: For EN-DC the NR RRCReconfiguration (Table 7.1.3.4.1.3.3-1 with cond EN-DC) and RadioBearerConfig message (Table 7.1.3.4.1.3.3-2) are contained in RRCConnectionReconfiguration 36.508 [7], Table 4.6.1-8 using conditions EN-DC_EmbedNR_RRCRecon, EN-DC_PSCell_HO and RBConfig_KeyChange. IE sk-Counter-r15 is included with a value incremented by 1 than previous value. Note 4: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. Note 5: For pc_supportOfRedCap_r17=false AND pc_supportOfERedCap_r18= false, the PDCP STATUS REPORT PDU size shall be 6 octets with 3 octets of RLC header, 2 octets of MAC header and 3 octets of MAC BSR or padding, so TBS of 112 bits shall be allocated. For pc_supportOfRedCap_r17=true OR pc_supportOfERedCap_r18=true, the PDCP STATUS REPORT PDU size shall be 6 octets with 2 octets of RLC header, 2 octets of MAC header and 4 octets of MAC BSR or padding, so TBS of 112 bits shall be allocated. Note 6: PDCP PDU with m = 2 is sent at t = 0 and the subsequent PDCP PDUs are sent at t = (m-2)*100. Note 7: Per TS 38.508-1 [4] Table 4.6.3-66: LogicalChannelConfig, both SRB1 and DRB have the same logical channel priority with prioritisedBitRate as infinity. Note 8 PDCP PDUs at step 15 (m=2 to 4) may be received by the SS in the same slot or in multiple slots (max one MAC PDU in a slot). Note 9 PDCP PDUs at steps 30 and 31 may be received by the SS in the same slot or in multiple slots (max one MAC PDU in a slot). |  |  |  |  |  |  |

7.1.3.4.1.3.3	Specific message contents

Table 7.1.3.4.1.3.3-0: SchedulingRequest-Config (Preamble)

| Derivation Path: TS 38.508-1 [4], Table: 4.6.3-155 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequestConfig ::= SEQUENCE { |  |  |  |
| schedulingRequestToAddModList SEQUENCE (SIZE(1..maxNrofSR-ConfigPerCellGroup)) OF SchedulingRequestToAddMod { | 1 entry |  |  |
| SchedulingRequestToAddMod[1] SEQUENCE { |  | entry 1 |  |
| sr-TransMax | n64 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.4.1.3.3-1: RRCReconfiguration (steps 8 and 23)

| Derivation Path: TS 38.508-1 [4], Table: 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig |  | NR |
| secondaryCellGroup | CellGroupConfig |  | EN-DC |
| nonCriticalExtension SEQUENCE { |  |  | NR |
| masterCellGroup | CellGroupConfig |  |  |
| masterKeyUpdate SEQUENCE { |  |  |  |
| keySetChangeIndicator | false | Horizontal key derivation |  |
| nextHopChainingCount | 0 |  |  |
| nas-Container | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.4.1.3.3-2: RadioBearerConfig (Table 7.1.3.4.1.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table: 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| drb-Identity | 2 | SCG DRB Id | EN-DC |
|  | Default DRB of the first PDU session |  | NR |
| reestablishPDCP | True |  |  |
| recoverPDCP | Not present |  |  |
| pdcp-Config | PDCP-Config |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.4.1.3.3-3: PDCP-Config (Table 7.1.3.4.1.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table: 4.6.3-99 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-Config ::= SEQUENCE { |  |  |  |
| drb SEQUENCE { |  |  |  |
| statusReportRequired | True |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.4.1.3.3-4: CellGroupConfig for EN-DC (Table 7.1.3.4.1.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table: 4.6.3-19 with condition PSCell_change |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| spCellConfigCommon | ServingCellConfigCommon |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.4.1.3.3-4A: CellGroupConfig for NR/5GC (Table 7.1.3.4.1.3.3-1)

Derivation Path: TS 38.508-1 [4], Table: 4.6.3-19 with condition PCell_change

Table 7.1.3.4.1.3.3-5: Void

Table 7.1.3.4.1.3.3-6: Void

##### 7.1.3.4.2	PDCP handover / Non-lossless handover / PDCP sequence number maintenance

7.1.3.4.2.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with default RB using RLC-UM }

ensure that {

when { UE is requested to make a non-lossless handover by SS }

then { UE transmits next PDCP Data PDU with SN value 0 }

}

(2)

with { UE in RRC_CONNECTED state with default RB using RLC-UM }

ensure that {

when { UE is requested to make a non-lossless handover by SS }

then { UE is able to receive next PDCP Data PDU with SN value 0 }

}

7.1.3.4.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.323, clause 5.1.2. Unless otherwise stated these are Rel-15 requirements.

[TS 38.323, clause 5.1.2]

When upper layers request a PDCP entity re-establishment, the UE shall additionally perform once the procedures described in this section. After performing the procedures in this section, the UE shall follow the procedures in subclause 5.2.

When upper layers request a PDCP entity re-establishment, the transmitting PDCP entity shall:

-	for UM DRBs and AM DRBs, reset the header compression protocol for uplink and start with an IR state in U-mode (as defined in RFC 3095 [8] and RFC 4815 [9]) if drb-ContinueROHC is not configured in TS 38.331 [3];

-	for UM DRBs and SRBs, set TX_NEXT to the initial value;

-	for SRBs, discard all stored PDCP SDUs and PDCP PDUs;

-	apply the ciphering algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	apply the integrity protection algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	for UM DRBs, for each PDCP SDU already associated with a PDCP SN but for which a corresponding PDU has not previously been submitted to lower layers:

-	consider the PDCP SDUs as received from upper layer;

-	perform transmission of the PDCP SDUs in ascending order of the COUNT value associated to the PDCP SDU prior to the PDCP re-establishment without restarting the discardTimer.

-	for AM DRBs, from the first PDCP SDU for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by lower layers, perform retransmission or transmission of all the PDCP SDUs already associated with PDCP SNs in ascending order of the COUNT values associated to the PDCP SDU prior to the PDCP entity re-establishment as specified below:

-	perform header compression of the PDCP SDU as specified in the subclause 5.7.4;

-	perform integrity protection and ciphering of the PDCP SDU using the COUNT value associated with this PDCP SDU as specified in the subclause 5.9 and 5.8;

-	submit the resulting PDCP Data PDU to lower layer.

When upper layers request a PDCP entity re-establishment, the receiving PDCP entity shall:

-	process the PDCP Data PDUs that are received from lower layers due to the re-establishment of the lower layers, as specified in the subclause 5.2.2.1;

-	for SRBs, discard all stored PDCP SDUs and PDCP PDUs;

-	for UM DRBs, if t-Reordering is running:

-	stop and reset t-Reordering;

-	deliver all stored PDCP SDUs to the upper layers in ascending order of associated COUNT values after performing header decompression.

-	for AM DRBs, perform header decompression for all stored PDCP SDUs if drb-ContinueROHC is not configured in TS 38.331 [3];

-	for UM DRBs and AM DRBs, reset the header compression protocol for downlink and start with NC state in U-mode (as defined in RFC 3095 [8] and RFC 4815 [9]) if drb-ContinueROHC is not configured in TS 38.331 [3];

-	for UM DRBs and SRBs, set RX_NEXT and RX_DELIV to the initial value;

-	apply the ciphering algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	apply the integrity protection algorithm and key provided by upper layers during the PDCP entity re-establishment procedure.

7.1.3.4.2.3	Test description

7.1.3.4.2.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.3.0 except that the DRB under test is configured in RLC UM mode. For EN-DC, ciphering algorithm is configured as null on E-UTRA.

7.1.3.4.2.3.2	Test procedure sequence

Table 7.1.3.4.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS creates 3 PDCP Data PDUs and the TX_NEXT is set to "0". | - | - | - | - |
| - | EXCEPTION: Step 2 and 3 shall be repeated for k=0 to 1 (increment=1). | - | - | - | - |
| 2 | The SS sends the PDCP Data PDU #k via RLC-UM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = k. After having sent a PDU, the SS set TX_NEXT= k+1. | <-- | PDCP PDU DATA #k | - | - |
| 3 | The UE sends the PDCP Data PDU #k via RLC-UM RB with the following content to the SS: D/C field = 1 (PDCP Data PDU) and PDCP SN = k. | --> | PDCP PDU DATA #k | - | - |
| 4 | The SS transmits NR RRCReconfiguration message to trigger non-lossless handover to the same SpCell. (Note 1, Note 3 and Note4) | <-- | RRCReconfiguration | - | - |
| 5 | The UE transmits a NR RRCReconfigurationComplete message. (Note 2) | --> | RRCReconfigurationComplete | - | - |
| 6 | The SS sends the PDCP Data PDU #2 via RLC-UM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 0. After having sent a PDU, the SS set TX_NEXT= 1. | <-- | PDCP PDU DATA #2 | - | - |
| 7 | Check: Does the UE send the PDCP Data PDU #2 via RLC-UM RB with the following content back to the SS: D/C field = 1 (PDCP Data PDU) and PDCP SN = 0? | --> | PDCP PDU DATA #2 | 1, 2 | P |
| Note 1: For EN-DC the NR RRCReconfiguration message with SCG Key change (secondary to master)is contained in RRCConnectionReconfiguration TS 36.508 [7], Table 4.6.1-8 using condition EN-DC_PSCell_HO and RBConfig_KeyChange. Note 2: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. Note 3: The RRCReconfiguration message triggers UE to perform the Random Access procedure, MAC reset, RLC and PDCP re-establishment. Note 4: For NR, the RRCReconfiguration message with master key change is as per RRCReconfiguration-HO with condition RBConfig_KeyChange according to TS 38.508-1 [4], Table 4.8.1-1A. |  |  |  |  |  |

7.1.3.4.2.3.3	Specific message contents

Table 7.1.3.4.2.3.3-1: RRCReconfiguration for EN-DC (step 4, Table 7.1.3.4.2.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition EN-DC_HO.

Table 7.1.3.4.2.3.3-1A: RRCReconfiguration for NR/5GC (step 4, Table 7.1.3.4.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.8.1-1A with condition RBConfig_KeyChange |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| masterKeyUpdate SEQUENCE { |  |  |  |
| keySetChangeIndicator | false | Horizontal key derivation |  |
| nextHopChainingCount | 0 |  |  |
| nas-Container | not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.4.2.3.3-2: RadioBearerConfig for EN-DC (step 4, Table 7.1.3.4.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-132 with condition EN-DC_DRB AND Re-establish_PDCP |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| securityConfig SEQUENCE { |  |  |  |
| securityAlgorithmConfig | SecurityAlgorithmConfig |  |  |
| keyToUse | master |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.4.2.3.3-3: SecurityAlgorithmConfig for EN-DC (Table 7.1.3.4.2.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-165 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SecurityAlgorithmConfig ::= SEQUENCE { |  |  |  |
| cipheringAlgorithm | nea0 |  |  |
| } |  |  |  |

Table 7.1.3.4.2.3.3-4: CellGroupConfig for EN-DC (step 4, Table 7.1.3.4.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with condition PSCell_change |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 1 entry |  | EN-DC |
| RLC-Bearer-Config[1] | RLC-BearerConfig with conditions UM and DRB2 and Re-establish_RLC | entry 1 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.4.2.3.3-5: CellGroupConfig for NR/5GC (step 4, Table 7.1.3.4.2.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with condition PCell_change

##### 7.1.3.4.3	PDCP handover / DAPS handover / Status reporting / Intra-frequency

7.1.3.4.3.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state and supporting Intra-frequency DAPS handover }

ensure that {

when { UE receives an RRCReconfiguration message including a reconfigurationWithSync for Intra-frequency DAPS handover }
    then { PDCP entity associated with a DAPS bearer shall keep DL/UL reception/transmission with the source gNB }

}

(2)

with { UE in NR RRC_CONNECTED state and supporting Intra-frequency DAPS handover and receiving an RRCReconfiguration message including a reconfigurationWithSync for Intra-frequency DAPS handover }

ensure that {

when { UE has performed random access procedure to the target cell successfully }
    then { UE shall perform uplink data switching }

}

(3)

with { UE in NR RRC_CONNECTED state and supporting Intra-frequency DAPS handover and receiving an RRCReconfiguration message including a reconfigurationWithSync for Intra-frequency DAPS handover }

ensure that {

when { upper layer requests a uplink data switching }
    then { UE shall send a PDCP status report for the DAPS bearer }

}

(4)

with { UE in NR RRC_CONNECTED state and supporting Intra-frequency DAPS handover and receiving an RRCReconfiguration message including a reconfigurationWithSync for Intra-frequency DAPS handover }

ensure that {

when { upper layer requests a PDCP entity reconfiguration and the associated RLC entity is released for a radio bearer }
    then { UE shall send a PDCP status report for the DAPS bearer }

}

7.1.3.4.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331 clause 5.3.5.5.2 and TS 38323 clause 4.2.2, 5.4.1, 5.8, 5.9 and 5.13. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.3.5.5.2]

The UE shall perform the following actions to execute a reconfiguration with sync.

1>	if the AS security is not activated, perform the actions upon going to RRC_IDLE as specified in 5.3.11 with the release cause 'other' upon which the procedure ends;

…

1>	If any DAPS bearer is configured:

2>	create a MAC entity for the target cell group with the same configuration as the MAC entity for the source cell group;

2>	for each DAPS bearer:

3>	establish an RLC entity or entities for the target cell group, with the same configurations as for the source cell group;

3>	establish the logical channel for the target cell group, with the same configurations as for the source cell group;

NOTE 2b:	In order to understand if a DAPS bearer is configured, the UE needs to check the presence of the field daps-Config within the RadioBearerConfig IE received in radioBearerConfig or radioBearerConfig2.

2>	for each SRB:

3>	establish an RLC entity for the target cell group, with the same configurations as for the source cell group;

3>	establish the logical channel for the target cell group, with the same configurations as for the source cell group;

3>	suspend SRBs for the source cell group;

NOTE 3:	Void

2>	apply the value of the newUE-Identity as the C-RNTI in the target cell group;

2>	configure lower layers for the target SpCell in accordance with the received spCellConfigCommon;

2>	configure lower layers for the target SpCell in accordance with any additional fields, not covered in the previous, if included in the received reconfigurationWithSync.

[TS 38.323, clause 4.2.2]

The PDCP entities are located in the PDCP sublayer. Several PDCP entities may be defined for a UE. Each PDCP entity is carrying the data of one radio bearer. A PDCP entity is associated either to the control plane or the user plane depending on which radio bearer it is carrying data for.

Figure 4.2.2.1 represents the functional view of the PDCP entity for the PDCP sublayer; it should not restrict implementation. The figure is based on the radio interface protocol architecture defined in TS 38.300 [2].

For split bearers and DAPS bearers, routing is performed in the transmitting PDCP entity.

A PDCP entity associated with DRB can be configured by upper layers TS 38.331 [3] to use header compression. In this version of the specification, the robust header compression protocol (ROHC) and the Ethernet header compression protocol (EHC) are supported. Each header compression protocol is independently configured for a DRB.

Figure 4.2.2-1: PDCP layer, functional view

Figure 4.2.2-2 represents the functional view of the PDCP entity associated with the DAPS bearer for the PDCP sublayer; it should not restrict implementation. The figure is based on the radio interface protocol architecture defined in TS 38.300 [2].

For DAPS bearers, the PDCP entity is configured with two sets of security functions and keys and two sets of header compression protocols.

Figure 4.2.2-2: PDCP layer associated with DAPS bearer, functional view

[TS 38.323, clause 5.4.1]

For AM DRBs configured by upper layers to send a PDCP status report in the uplink (statusReportRequired in TS 38.331 [3]), the receiving PDCP entity shall trigger a PDCP status report when:

-	upper layer requests a PDCP entity re-establishment;

-	upper layer requests a PDCP data recovery;

-	upper layer requests a uplink data switching;

-	upper layer reconfigures the PDCP entity to release DAPS and daps-SourceRelease is configured in TS 38.331 [3].

For UM DRBs configured by upper layers to send a PDCP status report in the uplink (statusReportRequired in TS 38.331 [3]), the receiving PDCP entity shall trigger a PDCP status report when:

-	upper layer requests a uplink data switching.

[TS 38.323, clause 5.8]

The ciphering function includes both ciphering and deciphering and is performed in PDCP, if configured. The data unit that is ciphered is the MAC-I (see clause 6.3.4) and the data part of the PDCP Data PDU (see clause 6.3.3) except the SDAP header and the SDAP Control PDU if included in the PDCP SDU. The ciphering is not applicable to PDCP Control PDUs.

For downlink and uplink, the ciphering algorithm and key to be used by the PDCP entity are configured by upper layers TS 38.331 [3] and the ciphering method shall be applied as specified in TS 33.501 [6].

The ciphering function is activated/suspended/resumed by upper layers TS 38.331 [3]. When security is activated and not suspended, the ciphering function shall be applied to all PDCP Data PDUs indicated by upper layers TS 38.331 [3] for the downlink and the uplink, respectively.

For DAPS bearers, the PDCP entity shall perform the ciphering or deciphering for the PDCP SDU using the ciphering algorithm and key either configured for the source cell or configured for the target cell, based on to/from which cell the PDCP SDU is transmitted/received.

For downlink and uplink ciphering and deciphering, the parameters that are required by PDCP for ciphering are defined in TS 33.501 [6] and are input to the ciphering algorithm. The required inputs to the ciphering function include the COUNT value, and DIRECTION (direction of the transmission: set as specified in TS 33.501 [6]). The parameters required by PDCP which are provided by upper layers TS 38.331 [3] are listed below:

-	BEARER (defined as the radio bearer identifier in TS 33.501 [6]. It will use the value RB identity –1 as in TS 38.331 [3]);

-	KEY (the ciphering keys for the control plane and for the user plane are KRRCenc and KUPenc, respectively).

[TS 38.323, clause 5.9]

The integrity protection function includes both integrity protection and integrity verification and is performed in PDCP, if configured. The data unit that is integrity protected is the PDU header and the data part of the PDU before ciphering. The integrity protection is always applied to PDCP Data PDUs of SRBs. The integrity protection is applied to sidelink SRB1, SRB2 and SRB3. The integrity protection is applied to PDCP Data PDUs of DRBs (including sidelink DRBs for unicast) for which integrity protection is configured. The integrity protection is not applicable to PDCP Control PDUs.

For downlink and uplink, the integrity protection algorithm and key to be used by the PDCP entity are configured by upper layers TS 38.331 [3] and the integrity protection method shall be applied as specified in TS 33.501 [6].

The integrity protection function is activated/suspended/resumed by upper layers TS 38.331 [3]. When security is activated and not suspended, the integrity protection function shall be applied to all PDUs including and subsequent to the PDU indicated by upper layers TS 38.331 [3] for the downlink and the uplink, respectively.

NOTE 1:	As the RRC message which activates the integrity protection function is itself integrity protected with the configuration included in this RRC message, this message needs first be decoded by RRC before the integrity protection verification could be performed for the PDU in which the message was received.

NOTE 2:	As the PC5-S message which activates the integrity protection function is itself integrity protected with the configuration included in this PC5-S message, this message needs first be decoded by upper layer before the integrity protection verification could be performed for the PDU in which the message was received.

For DAPS bearers, the PDCP entity shall perform the integrity protection or verification for the PDCP SDU using the integrity protection algorithm and key either configured for the source cell or configured for the target cell, based on to/from which cell the PDCP SDU is transmitted/received.

For downlink and uplink integrity protection and verification, the parameters that are required by PDCP for integrity protection are defined in TS 33.501 [6] and are input to the integrity protection algorithm. The required inputs to the integrity protection function include the COUNT value, and DIRECTION (direction of the transmission: set as specified in TS 33.501 [6]). The parameters required by PDCP which are provided by upper layers TS 38.331 [3] are listed below:

-	BEARER (defined as the radio bearer identifier in TS 33.501 [6]. It will use the value RB identity –1 as in TS 38.331 [3]);

-	KEY (the integrity protection keys for the control plane and for the user plane are KRRCint and KUPint, respectively).

[TS 38.323, clause 5.13]

For DAPS bearers, when upper layers request uplink data switching, the transmitting PDCP entity shall:

-	for AM DRBs, from the first PDCP SDU for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by the RLC entity associated with the source cell, perform retransmission or transmission of all the PDCP SDUs already associated with PDCP SNs in ascending order of the COUNT values associated to the PDCP SDU prior to uplink data switching to the RLC entity associated with the target cell as specified below:

-	perform header compression of the PDCP SDU using ROHC as specified in the clause 5.7.4;

-	perform integrity protection and ciphering of the PDCP SDU using the COUNT value associated with this PDCP SDU as specified in the clause 5.9 and 5.8;

-	submit the resulting PDCP Data PDU to lower layer, as specified in clause 5.2.1.

-	for UM DRBs, for all PDCP SDUs which have been processed by PDCP but which have not yet been submitted to lower layers, perform transmission of the PDCP SDUs in ascending order of the COUNT values to the RLC entity associated with the target cell as specified below:

-	perform header compression of the PDCP SDU using ROHC as specified in the clause 5.7.4;

-	perform integrity protection and ciphering of the PDCP SDU using the COUNT value associated with this PDCP SDU as specified in the clause 5.9 and 5.8;

-	submit the resulting PDCP Data PDU to lower layer, as specified in clause 5.2.1.

7.1.3.4.3.3	Test description

7.1.3.4.3.3.1	Pre-test conditions

System Simulator:

Same Pre-test conditions as in clause 7.1.3.0 except the following:

-	NR Cell 1 is the Serving cell and the power level is configured to ''Serving Cell” defined in TS 38.508-1 [4] Table 6.2.2.1-3.

-	NR Cell 2 is the Suitable neighbour intra-frequency cell and the power level is configured to '' Suitable neighbour intra-frequency cell” defined in TS 38.508-1 [4] Table 6.2.2.1-3.

-	System information combination NR-2 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used for both NR Cells.

-	Test Loop Function (On) with UE test loop mode A (message condition UE TEST LOOP MODE A to return one UL PDCP SDU per DL PDCP SDU) activated according to TS 38.508-1 [4], table 4.5.4.2-3.

UE:

-	None.

Preamble:

-	The UE is in 3N-A state configured with DRB#1 in RLC AM mode according to TS 38.508-1 [4], Table 4.4A.2-3.

7.1.3.4.3.3.2	Test procedure sequence

Table 7.1.3.4.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message containing reconfigurationWithSync to order UE to perform DAPS handover to NR Cell 2. DRB#n of the first PDU session is configured as DAPS bearer. | <-- | NR RRC: RRCReconfiguration | - | - |
| - | EXCEPTION: In parallel with step 2-3, parallel behaviour defined in table 7.1.3.4.3.3.2-2 is executed repeatedly. | - | - | - | - |
| 2 | The SS sends one IP Packet to the UE via DRB#n of the first PDU session in NR Cell 1 and | <-- | PDCP PDU DATA #0 | - | - |
| 2A | The SS stops sending RLC acknowledgements. | - | - | - | - |
| 3 | Check: Does the UE loop back the IP packet received at step 2 in NR Cell 1? | --> | PDCP PDU DATA #0 | 1 | P |
| 4 | The SS transmits Random Access Response to respond to the latest preamble in NR Cell 2. | <- | Random Access Response | - | - |
| - | EXCEPTION: Steps 5-7 can occur in any order. | - | - | - | - |
| 5 | The UE transmits an RRCReconfigurationComplete message in NR Cell 2. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 6 | Check: Does the UE retransmit the IP packet received at step 2 in NR Cell 2? | --> | PDCP PDU DATA #0 | 2 | P |
| 7 | Check: Does the UE send PDCP status report in NR Cell 2? (Note 1) | --> | PDCP STATUS REPORT | 3 | P |
| 8 | The SS transmits an RRCReconfiguration message with condition DAPS_HO_ReleaseSource in NR Cell 2. | <-- | NR RRC: RRCReconfiguration | - | - |
| - | EXCEPTION: Steps 9-10 can occur in any order. | - | - | - | - |
| 9 | The UE transmits an RRCReconfigurationComplete message in NR Cell 2. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 10 | Check: Does the UE send PDCP status report in NR Cell 2? (Note 1) | --> | PDCP STATUS REPORT | 4 | P |
| Note 1: D/C field = 0 (PDCP control PDU) and PDU Type =000 (PDCP status report), FMC field = 1. |  |  |  |  |  |

Table 7.1.3.4.3.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits preamble to NR Cell 2. | -> | (PRACH Preamble) | - | - |

7.1.3.4.3.3.3	Specific message contents

Table 7.1.3.4.3.3.3-1: RRCReconfiguration (step 1, Table 7.1.3.4.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.8.1-1A with Condition RBConfig_NoKeyChange |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig with conditions DRBn and DAPS_PDCP |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.4.3.3.3-2: CellGroupConfig (Table 7.1.3.4.3.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 1 entry |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions AM, DRBn | entry1 |  |
| } |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| spCellConfigCommon SEQUENCE { |  |  |  |
| physCellId | Physical Cell Identity of NR Cell 2 |  |  |
| uplinkConfigCommon | UplinkConfigCommon | Table 7.1.3.4.3.3.3-5 |  |
| } |  |  |  |
| rach-ConfigDedicated CHOICE { |  |  |  |
| uplink | RACH-ConfigDedicated | OCTET STRING (CONTAINING RACH-ConfigDedicated) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.4.3.3.3-3: RACH-ConfigDedicated (Table 7.1.3.4.3.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-129 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigDedicated ::= SEQUENCE { |  |  |  |
| cfra SEQUENCE { |  |  |  |
| occasions SEQUENCE { |  |  |  |
| rach-ConfigGeneric | RACH-ConfigGeneric | OCTET STRING (CONTAINING RACH-ConfigGeneric) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.4.3.3.3-4: RACH-ConfigGeneric (Table 7.1.3.4.3.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| preambleTransMax | n200 |  |  |
| } |  |  |  |

Table 7.1.3.4.3.3.3-5: UplinkConfigCommon (Table 7.1.3.4.3.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-201 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkConfigCommon ::= SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkCommon | Table 7.1.3.4.3.3.3-6 |  |
| } |  |  |  |

Table 7.1.3.4.3.3.3-6: BWP-UplinkCommon  (Table 7.1.3.4.3.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkCommon ::= SEQUENCE { |  |  |  |
| rach-ConfigCommon CHOICE { |  |  |  |
| setup | RACH-ConfigCommon | Table 7.1.3.4.3.3.3-7 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.4.3.3.3-7: RACH-ConfigCommon (Table 7.1.3.4.3.3.3-6)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-128 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon::= SEQUENCE { |  |  |  |
| rach-ConfigGeneric | RACH-ConfigGeneric | Table 7.1.3.4.3.3.3-8 |  |
| } |  |  |  |

Table 7.1.3.4.3.3.3-8: RACH-ConfigGeneric (Table 7.1.3.4.3.3.3-7)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| preambleTransMax | 200 |  |  |
| } |  |  |  |

Table 7.1.3.4.3.3.3-9: RRCReconfiguration (step 8, Table 7.1.3.4.3.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.8.1-1A with Condition DAPS_HO_ReleaseSource

##### 7.1.3.4.4	PDCP handover / DAPS handover / Status reporting / Inter-frequency

###### 7.1.3.4.4.1	PDCP handover / DAPS handover / Status reporting / Inter-frequency / Intra-band non-contiguous

7.1.3.4.4.1.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state and supporting Inter-frequency DAPS handover }

ensure that {

when { UE receives an RRCReconfiguration message including a reconfigurationWithSync for Inter-frequency DAPS handover }

then { PDCP entity associated with a DAPS bearer shall keep DL/UL reception/transmission with the source gNB }

}

(2)

with { UE in NR RRC_CONNECTED state and supporting Inter-frequency DAPS handover and receiving an RRCReconfiguration message including a reconfigurationWithSync for Inter-frequency DAPS handover }

ensure that {

when { UE has performed random access procedure to the target cell successfully }

then { UE shall perform uplink data switching }

}

(3)

with { UE in NR RRC_CONNECTED state and supporting Inter-frequency DAPS handover and receiving an RRCReconfiguration message including a reconfigurationWithSync for Inter-frequency DAPS handover }

ensure that {

when { upper layer requests a uplink data switching }

then { UE shall send a PDCP status report for the DAPS bearer }

}

(4)

with { UE in NR RRC_CONNECTED state and supporting Inter-frequency DAPS handover and receiving an RRCReconfiguration message including a reconfigurationWithSync for Inter-frequency DAPS handover }

ensure that {

when { upper layer requests a PDCP entity reconfiguration and the associated RLC entity is released for a radio bearer }

then { UE shall send a PDCP status report for the DAPS bearer }

}

7.1.3.4.4.1.2	Conformance requirements

Same as test case 7.1.3.4.3.2.

7.1.3.4.4.1.3	Test description

7.1.3.4.4.1.3.1	Pre-test conditions

Same as test case 7.1.3.4.3 with the following differences:

-	Cells configuration: NR Cell 3 replaces NR Cell 2.

-	System information combination: NR-4 replaces NR-2.

7.1.3.4.4.1.3.2	Test procedure sequence

Same as test case 7.1.3.4.3 with the following differences:

-	Cells configuration: NR Cell 3 replaces NR Cell 2.

7.1.3.4.4.1.3.3	Specific message contents

Same as test case 7.1.3.4.3 with the following differences:

-	Cells configuration: NR Cell 3 replaces NR Cell 2.

###### 7.1.3.4.4.2	PDCP handover / DAPS handover / Status reporting / Inter-frequency / Intra-band contiguous

7.1.3.4.4.2.1	Test Purpose (TP)

Same as test case 7.1.3.4.4.1.1

7.1.3.4.4.2.2	Conformance requirements

Same as test case 7.1.3.4.3.2.

7.1.3.4.4.2.3	Test description

7.1.3.4.4.2.3.1	Pre-test conditions

Same as test case 7.1.3.4.3 with the following differences:

-	Cells configuration: NR Cell 3 replaces NR Cell 2.

-	System information combination: NR-4 replaces NR-2.

-	Test frequency of NR Cell 1 and NR Cell 3 is configured according to TS 38.508-1[4] Table 6.2.3.4-1.

7.1.3.4.4.2.3.2	Test procedure sequence

Same as test case 7.1.3.4.3 with the following differences:

-	Cells configuration: NR Cell 3 replaces NR Cell 2.

7.1.3.4.4.2.3.3	Specific message contents

Same as test case 7.1.3.4.3 with the following differences:

-	Cells configuration: NR Cell 3 replaces NR Cell 2.

#### 7.1.3.5	PDCP other

##### 7.1.3.5.1	PDCP Discard

7.1.3.5.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { the Discard Timer for a PDCP SDU expires }

then { UE discards the corresponding PDCP SDU }

}

7.1.3.5.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.323, clause 5.3. Unless otherwise stated these are Rel-15 requirements.

[TS 38.323, clause 5.3]

When the discardTimer expires for a PDCP SDU, or the successful delivery of a PDCP SDU is confirmed by PDCP status report, the transmitting PDCP entity shall discard the PDCP SDU along with the corresponding PDCP Data PDU. If the corresponding PDCP Data PDU has already been submitted to lower layers, the discard is indicated to lower layers.

For SRBs, when upper layers request a PDCP SDU discard, the PDCP entity shall discard all stored PDCP SDUs and PDCP PDUs.

NOTE:	Discarding a PDCP SDU already associated with a PDCP SN causes a SN gap in the transmitted PDCP Data PDUs, which increases PDCP reordering delay in the receiving PDCP entity. It is up to UE implementation how to minimize SN gap after SDU discard.

[TS 38.323, clause 7.1]

This sub clause describes the state variables used in PDCP entities in order to specify the PDCP protocol. The state variables defined in this subclause are normative.

All state variables are non-negative integers, and take values from 0 to [232 – 1].

PDCP Data PDUs are numbered integer sequence numbers (SN) cycling through the field: 0 to [2[pdcp-SN-Size] – 1].

The transmitting PDCP entity shall maintain the following state variables:

a)	TX_NEXT

This state variable indicates the COUNT value of the next PDCP SDU to be transmitted. The initial value is 0.

The receiving PDCP entity shall maintain the following state variables:

a)	RX_NEXT

This state variable indicates the COUNT value of the next PDCP SDU expected to be received. The initial value is 0.

b)	RX_DELIV

This state variable indicates the COUNT value of the first PDCP SDU not delivered to the upper layers, but still waited for. The initial value is 0.

c)	RX_REORD

This state variable indicates the COUNT value following the COUNT value associated with the PDCP Data PDU which triggered t-Reordering.

[TS 38.323, clause 6.3.5]

Length: 32 bits

The COUNT value is composed of a HFN and the PDCP SN. The size of the HFN part in bits is equal to 32 minus the length of the PDCP SN.

Figure 6.3.5-1: Format of COUNT

NOTE:	COUNT does not wrap around.

7.1.3.5.1.3	Test description

7.1.3.5.1.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.3.0 with exceptions listed in Table 7.1.3.5.1.3.1-1 applicable for the configured UM DRB and Table 7.1.3.5.1.3.3-1 for SR configuration except that PDCP is configured for 18 bit SN.

Table 7.1.3.5.1.3.1-1: PDCP Settings

| Parameter | Value |
| --- | --- |
| Discard_Timer | 500 ms |

7.1.3.5.1.3.2	Test procedure sequence

Table 7.1.3.5.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: The SS does not allocate UL grants unless when explicitly stated so in the procedure. | - | - | - | - |
| 1 | The SS creates 5 PDCP Data PDUs and the PDCP SN = "0" within TX_NEXT. |  | - | - | - |
| 2 | Void |  |  |  |  |
| - | EXCEPTION: Step 3 shall be repeated for k=0 to 2 (increment=1) with the below specified PDU size sent to the UE: Data PDU#1 = 46 bytes for k=0 Data PDU#2 = 62 bytes for k=1 Data PDU#3 = 78 bytes for k=2 | - | - | - | - |
| 3 | The SS sends a PDCP Data PDU via RLC-UM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = k After having sent a PDU, the SS sets PDCP SN is set to k+1 within TX_NEXT. | <-- | PDCP DATA PDU (SN=k) | - | - |
| 4 | Wait for Discard_Timer to expire. Note: According to TS38.508-1, timer tolerance should be 10% of Discard_Timer. | - | - | - | - |
| - | EXCEPTION: Step 5 shall be repeated for k=3 to 4 (increment=1) with the below specified PDU size sent to the UE: Data PDU#4 = 94 bytes for k=3 Data PDU#5 = 110 bytes for k=4 | - | - | - | - |
| 5 | The SS sends a PDCP Data PDU via RLC-UM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = k After having sent a PDU, the SS sets PDCP SN is set to k+1 within TX_NEXT. | <-- | PDCP DATA PDU (SN=k) | - | - |
| 6 | The SS resumes normal UL grant allocation. | - | - | - | - |
| 7 | Check: Does UE transmit a PDCP Data PDU # 4 of size 94 bytes? (Note1) (Note 2) | --> | PDCP Data PDU # 4 | 1 | P |
| 8 | Check: Does UE transmit a PDCP Data PDU # 5 of size 110 bytes? (Note1) (Note 2) | --> | PDCP Data PDU # 5 | 1 | P |
| Note 1 PDCP Data PDU contents are checked to verify that the UL PDU is same as the DL PDU. According to the Note in TS 38.323 [19] clause 5.3 in case of PDCP SDUs being discarded it is up to the UE implementation which SN to be used and therefore the SN cannot be checked. Note 2 PDCP PDUs at steps 7 and 8 may be received by the SS in the same slot or in multiple slots (max one MAC PDU in a slot). |  |  |  |  |  |

7.1.3.5.1.3.3	Specific message contents

Table 7.1.3.5.1.3.3-1: SchedulingRequest-Config (Preamble)

| Derivation Path: 38.508-1 [4], Table 4.6.3-155 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| sr-TransMax | n64 |  |  |

##### 7.1.3.5.1a	NR NTN / PDCP Discard / discardTimerExt2 configured

7.1.3.5.1a.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state on a cell which provides access by NR NTN }

ensure that {

when { discardTimerExt2 is configured }

then { UE discards the PDCP SDU considering value signalled in discardTimerExt2 and ignores discardTimer }

}

7.1.3.5.1a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.323, clause 5.3, 7.1, 6.3.5, 7.3 and TS 38.331, clause 6.3.2. Unless otherwise stated these are Rel-17 requirements.

[TS 38.323, clause 5.3]

When the discardTimer expires for a PDCP SDU, or the successful delivery of a PDCP SDU is confirmed by PDCP status report, the transmitting PDCP entity shall discard the PDCP SDU along with the corresponding PDCP Data PDU. If the corresponding PDCP Data PDU has already been submitted to lower layers, the discard is indicated to lower layers.

For SRBs, when upper layers request a PDCP SDU discard, the PDCP entity shall discard all stored PDCP SDUs and PDCP PDUs.

NOTE:	Discarding a PDCP SDU already associated with a PDCP SN causes a SN gap in the transmitted PDCP Data PDUs, which increases PDCP reordering delay in the receiving PDCP entity. It is up to UE implementation how to minimize SN gap after SDU discard.

[TS 38.323, clause 7.1]

This clause describes the state variables used in PDCP entities in order to specify the PDCP protocol. The state variables defined in this clause are normative.

All state variables are non-negative integers, and take values from 0 to [232 – 1].

PDCP Data PDUs are numbered integer sequence numbers (SN) cycling through the field: 0 to [2[pdcp-SN-SizeUL] – 1] or 0 to [2[pdcp-SN-SizeDL] – 1] or 0 to [2[sl-PDCP-SN-Size] – 1].

The transmitting PDCP entity shall maintain the following state variables:

a)	TX_NEXT

This state variable indicates the COUNT value of the next PDCP SDU to be transmitted. The initial value is 0, except for SRBs configured with state variables continuation. For target SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding source SRB. For source SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding target SRB.

The receiving PDCP entity shall maintain the following state variables:

a)	RX_NEXT

This state variable indicates the COUNT value of the next PDCP SDU expected to be received. The initial value is 0, except for sidelink broadcast and groupcast, for SRBs configured with state variables continuation, and for broadcast MRBs. For NR sidelink communication for broadcast and groupcast or sidelink SRB4 for NR sidelink discovery, the initial value of the SN part of RX_NEXT is (x +1) modulo (2[sl-PDCP-SN-Size]), where x is the SN of the first received PDCP Data PDU. For broadcast MRBs, the initial value of the SN part of RX_NEXT is (x +1) modulo (2[PDCP-SN-SizeDL]), where x is the SN of the first received PDCP Data PDU. For target SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding source SRB. For source SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding target SRB.

NOTE 1:	For NR sidelink communication for broadcast and groupcast or sidelink SRB4 for NR sidelink discovery, it is up to UE implementation to select the HFN part for RX_NEXT such that initial value of RX_DELIV should be a positive value.

NOTE 2:	For broadcast MRBs, the initial value of the HFN part of RX_NEXT is set by UE implementation.

b)	RX_DELIV

This state variable indicates the COUNT value of the first PDCP SDU not delivered to the upper layers, but still waited for. The initial value is 0, except for sidelink broadcast and groupcast, for SRBs configured with state variables continuation, and for MRBs. For NR sidelink communication for broadcast and groupcast or sidelink SRB4 for NR sidelink discovery, the initial value of the SN part of RX_DELIV is (x – 0.5 × 2[sl-PDCP-SN-Size–1]) modulo (2[sl-PDCP-SN-Size]), where x is the SN of the first received PDCP Data PDU. For broadcast MRBs, the initial value of the SN part of RX_DELIV is set to (x – 0.5 × 2[PDCP-SN-SizeDL–1]) modulo (2[PDCP-SN-SizeDL]), where x is the SN of the first received PDCP Data PDU. For multicast MRBs, the initial value of RX_DELIV is set, if provided, by initialRX-DELIV in TS 38.331 [3]. For target SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding source SRB. For source SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding target SRB.

NOTE 3:	For broadcast MRBs, the initial value of the HFN part of RX_DELIV is set by UE implementation.

c)	RX_REORD

This state variable indicates the COUNT value following the COUNT value associated with the PDCP Data PDU which triggered t-Reordering. For target SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding source SRB. For source SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding target SRB.

[TS 38.323, clause 6.3.5]

Length: 32 bits

The COUNT value is composed of a HFN and the PDCP SN. The size of the HFN part in bits is equal to 32 minus the length of the PDCP SN.

Figure 6.3.5-1: Format of COUNT

NOTE:	COUNT does not wrap around.

[TS 38.323, clause 7.3]

The transmitting PDCP entity shall maintain the following timers:

a) discardTimer

This timer is configured only for DRBs. The duration of the timer is configured by upper layers TS 38.331 [3]. In the transmitter, a new timer is started upon reception of an SDU from upper layer.

[TS 38.331, clause 6.3.2]

discardTimerExt2

Value in ms of discardTimerExt specified in TS 38.323 [5]. Value ms2000 corresponds to 2000 ms. If this field is present, the field discardTimer and discardTimerExt are ignored and discardTimerExt2 is used instead.

7.1.3.5.1a.3	Test description

7.1.3.5.1a.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.3.0 with exceptions listed in Table 7.1.3.5.1a.3.1-1 applicable for the configured UM DRB and Table 7.1.3.5.1a.3.3-1 for SR configuration.

Table 7.1.3.5.1a.3.1-1: PDCP Settings

| Parameter | Value |
| --- | --- |
| discardTimer | 500 ms |
| discardTimerExt2-r17 | 2000ms |

System Simulator:

-	NR Cell 1 as specified in TS 38.508-1 [4] Table 4.4.2.1.

-	System information combination NR-29 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used.

UE:

-	The pre-configured UE location is defined in TS 38.508-1 [4] Clause 4.5C.

7.1.3.5.1a.3.2	Test procedure sequence

Table 7.1.3.5.1a.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: The SS does not allocate UL grants unless when explicitly stated so in the procedure. | - | - | - | - |
| 1 | The SS sends a PDCP Data PDU via RLC-UM RB. Data PDU = 46 bytes. | <-- | PDCP DATA PDU (SN=0) | - | - |
| 2 | Wait for discardTimer to expire. Note: According to TS 38.508-1 [4], timer tolerance should be 10% of discardTimer. | - | - | - | - |
| 3 | The SS resumes normal UL grant allocation. | - | - | - | - |
| 4 | Check: Does UE transmit a PDCP Data PDU of size 46 bytes? (Note1) | --> | PDCP Data PDU | 1 | P |
| 5 | Configure SS not to allocate UL grant to the UE. | - | - | - | - |
| 6 | The SS sends a PDCP Data PDU via RLC-UM RB. Data PDU = 62 bytes. | <-- | PDCP DATA PDU (SN=1) | - | - |
| 7 | Wait for discardTimerExt2-r17 to expire. Note: According to TS 38.508-1 [4], timer tolerance should be 10% of discardTimerExt2-r17. | - | - | - | - |
| 8 | The SS sends a PDCP Data PDU via RLC-UM RB. Data PDU = 94 bytes. | <-- | PDCP DATA PDU (SN=2) | - | - |
| 9 | The SS resumes normal UL grant allocation. | - | - | - | - |
| 10 | Check: Does UE transmit a PDCP Data PDU of size 94 bytes? (Note1) | --> | PDCP Data PDU | 1 | P |
| Note 1 PDCP Data PDU contents are checked to verify that the UL PDU is same as the DL PDU. According to the Note in TS 38.323 [19] clause 5.3 in case of PDCP SDUs being discarded it is up to the UE implementation which SN to be used and therefore the SN cannot be checked. |  |  |  |  |  |

7.1.3.5.1a.3.3	Specific message contents

Table 7.1.3.5.1a.3.3-1: SchedulingRequest-Config (Preamble)

| Derivation Path: 38.508-1 [4], Table 4.6.3-155 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| sr-ProhibitTimer | ms64 |  |  |
| sr-TransMax | n64 |  |  |

##### 7.1.3.5.2	PDCP Uplink Routing / Split DRB

7.1.3.5.2.1	Test Purpose

(1)

with { UE in RRC_CONNECTED state with SCG activated with a Split DRB established and total amount of PDCP data volume is less than ul-DataSplitThreshold and not yet transmitted RLC data volume in the two associated RLC entities }

ensure that {

when { UE has PDCP SDUs available for transmission }

then { the UE transmits the PDCP SDUs on the Primary RLC entity }

}

(2)

with { UE in RRC_CONNECTED state with SCG activated with a Split DRB established and total amount of PDCP data volume is not less than ul-DataSplitThreshold and not yet transmitted RLC data volume in the two associated RLC entities }

ensure that {

when { UE has PDCP SDUs available for transmission }

then { the UE transmits the PDCP SDUs on the primary or secondary RLC entity }

}

7.1.3.5.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.323, clause 5.2.1. Unless otherwise stated these are Rel-15 requirements.

[TS 38.323, clause 5.2.1]

At reception of a PDCP SDU from upper layers, the transmitting PDCP entity shall:

-	start the discardTimer associated with this PDCP SDU (if configured).

For a PDCP SDU received from upper layers, the transmitting PDCP entity shall:

-	associate the COUNT value corresponding to TX_NEXT to this PDCP SDU;

NOTE 1:	Associating more than half of the PDCP SN space of contiguous PDCP SDUs with PDCP SNs, when e.g., the PDCP SDUs are discarded or transmitted without acknowledgement, may cause HFN desynchronization problem. How to prevent HFN desynchronization problem is left up to UE implementation.

-	perform header compression of the PDCP SDU as specified in the subclause 5.7.4;

-	perform integrity protection, and ciphering using the TX_NEXT as specified in the subclause 5.9 and 5.8, respectively;

-	set the PDCP SN of the PDCP Data PDU to TX_NEXT modulo 2[pdcp-SN-Size];

-	increment TX_NEXT by one;

-	submit the resulting PDCP Data PDU to lower layer as specified below.

When submitting a PDCP Data PDU to lower layer, the transmitting PDCP entity shall:

-	if the transmitting PDCP entity is associated with one RLC entity:

-	submit the PDCP Data PDU to the associated RLC entity.

-	else, if the transmitting PDCP entity is associated with two RLC entities:

-	if the PDCP duplication is activated:

-	if the PDCP PDU is a PDCP Data PDU:

-	duplicate the PDCP Data PDU and submit the PDCP Data PDU to both associated RLC entities.

-	else:

-	if the two associated RLC entities belong to the different Cell Groups; and

-	if the total amount of PDCP data volume and RLC data volume pending for initial transmission (as specified in TS 36.322 [5]) in the two associated RLC entities is equal to or larger than ul-DataSplitThreshold:

-	submit the PDCP Data PDU to either the primary RLC entity or the secondary RLC entity;.

-	else:

-	submit the PDCP Data PDU to the primary RLC entity.

NOTE 2:	If the transmitting PDCP entity is associated with two RLC entities, the UE should minimize the amount of PDCP PDUs submitted to lower layers before receiving request from lower layers and minimize the PDCP SN gap between PDCP PDUs submitted to two associated RLC entities to minimize PDCP reordering delay in the receiving PDCP entity.

7.1.3.5.2.3	Test description

7.1.3.5.2.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.3.0 with exceptions listed in Table 7.1.3.5.2.3.1-1 and Generic procedure parameter DC bearer(MCG and split).

For NR/5GC, NR Cell 1 is the PCell and NR Cell 10 is the PSCell and same Pre-test conditions as in clause 7.1.3.0 using generic procedure parameter Connectivity (NR-DC) with DC bearer(MCG and split).

Table 7.1.3.5.2.3.1-1: PDCP Settings

| Parameter | Value |
| --- | --- |
| Discard_Timer | 500 ms |
| ul-DataSplitThreshold | b100 |

7.1.3.5.2.3.2	Test procedure sequence

Table 7.1.3.5.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0A | SS transmits NR RRCReconfiguration message to configure the primary path on the MCG Cell. | <-- | RRCReconfiguration | - | - |
| 0B | UE transmits RRCReconfigurationComplete (Note 2). | --> | RRCReconfigurationComplete | - | - |
| 1 | The SS sends a PDCP Data PDU on the split DRB on AM RLC entity configured for SCG on PSCell. Data PDU = 64 bytes. | <-- | PDCP DATA PDU | - | - |
| - | EXCEPTION: In parallel with step 2, UE may execute parallel behaviour defined in table 7.1.3.5.2.3.2-2. | - | - | - | - |
| 2 | Check: Does UE transmit a PDCP Data PDU on the AM RLC primary entity? | --> | PDCP DATA PDU | 1 | P |
| 2A | SS transmits NR RRCReconfiguration message to configure the primary path on the SCG Cell. | <-- | RRCReconfiguration | - | - |
| 2B | UE transmits RRCReconfigurationComplete (Note 2). | --> | RRCReconfigurationComplete | - | - |
| 2C | The SS sends a PDCP Data PDU on the split DRB on AM RLC entity configured for SCG on PSCell. Data PDU = 64 bytes. | <-- | PDCP DATA PDU | - | - |
| - | EXCEPTION: In parallel with step 2D, UE may execute parallel behaviour defined in table 7.1.3.5.2.3.2-2. | - | - | - | - |
| 2D | Check: Does UE transmit a PDCP Data PDU on the primary AM RLC entity? | --> | PDCP DATA PDU | 1 | P |
| 3 | The SS sends a PDCP Data PDU on the split DRB on AM RLC entity configured for SCG on PSCell. Data PDU = 164 bytes. | <-- | PDCP DATA PDU | - | - |
| - | EXCEPTION: Steps 4a1 to 4b2 describe optional behaviour that depends on the UE uplink path | - | - | - | - |
| 4a1 | Check: Does UE transmit a PDCP Data PDU on the AM RLC primary entity? | --> | PDCP DATA PDU | 2 | P |
| 4a2 | Check: Does UE transmit a PDCP Data PDU on the AM RLC secondary entity | --> | PDCP DATA PDU | 2 | F |
| 4b1 | Check: Does UE transmit a PDCP Data PDU on the AM RLC secondary entity? | --> | PDCP DATA PDU | 2 | P |
| 4b2 | Check: Does UE transmit a PDCP Data PDU on the AM RLC primary entity? | --> | PDCP DATA PDU | 2 | F |
| 5 | SS transmits NR RRCReconfiguration message to configure new split DRB parameters, where the ul-DataSplitThreshold is equal to 0 bytes (Note 1). | <-- | RRCReconfiguration | - | - |
| 6 | UE transmits RRCReconfigurationComplete (Note 2). | --> | RRCReconfigurationComplete | - | - |
| 7 | The SS sends a PDCP Data PDU on the split DRB on AM RLC entity configured for SCG on PSCell. Data PDU = 64 bytes | <-- | PDCP Data PDU | - | - |
| - | EXCEPTION: Steps 8a1 to 8b2 describe optional behaviour that depends on the UE uplink path | - | - | - | - |
| 8a1 | Check: Does UE transmit a PDCP Data PDU on the AM RLC primary entity? | --> | PDCP DATA PDU | 2 | P |
| 8a2 | Check: Does UE transmit a PDCP Data PDU on the AM RLC secondary entity? | --> | PDCP DATA PDU | 2 | F |
| 8b1 | Check: Does UE transmit a PDCP Data PDU on the AM RLC secondary? | --> | PDCP DATA PDU | 2 | P |
| 8b2 | Check: Does UE transmit a PDCP Data PDU on the AM RLC primary? | --> | PDCP DATA PDU | 2 | F |
| Note 1: For EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration. Note 2: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. |  |  |  |  |  |

Table 7.1.3.5.2.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does UE transmit a PDCP Data PDU on the AM RLC secondary entity in next 2 seconds? NOTE: 2 seconds is sufficient time to discard PDCP PDU. | --> | PDCP DATA PDU | 1 | F |

7.1.3.5.2.3.3	Specific message contents

Table 7.1.3.5.2.3.3-0A: RRCConnectionReconfiguration (step 0A, step 2A, step 5, Table 7.1.3.5.3.3.2-1)

| Derivation Path: 36.508 [7], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nr-RadioBearerConfig1-r15 | OCTET STRING including RadioBearerConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.5.2.3.3-0B: RRCReconfiguration (step 0A, step 2C, step 5, Table 7.1.3.5.2.3.2-1)

| Derivation Path: 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| radioBearerConfig2 | OCTET STRING including RadioBearerConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.5.2.3.3-1: RadioBearerConfig (Table 7.1.3.5.2.3.3-0A and Table 7.1.3.5.2.3.3-0B)

| Derivation Path: 38.508-1 [4], Table 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| pdcp-Config SEQUENCE { |  |  |  |
| moreThanOneRLC SEQUENCE { |  |  |  |
| primaryPath SEQUENCE { |  |  |  |
| cellGroup | 0 |  | Step 0A |
|  | 1 |  | Step 2A |
| logicalChannel | LogicalChannelIdentity |  |  |
| } |  |  |  |
| ul-DataSplitThreshold | b0 |  | Step 5 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 7.1.3.5.3	PDCP Data Recovery

7.1.3.5.3.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with a DRB established using RLC-AM }

ensure that {

when { network requests reconfiguration and recovery of the DRB (without handover) }

then { UE reconfigures the DRB and performs retransmission of all the PDCP PDUs previously submitted to re-established AM RLC entity in ascending order of the associated COUNT values from the first PDCP PDU for which the successful delivery has not been confirmed by lower layers }

}

7.1.3.5.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.323, clauses 5.2.1, 5.4.1 and 5.5; TS 38.331, clause 5.3.5.4.3. Unless otherwise stated these are Rel-15 requirements.

[TS 38.323, clause 5.2.1]

At reception of a PDCP SDU from upper layers, the transmitting PDCP entity shall:

-	start the discardTimer associated with this PDCP SDU (if configured).

For a PDCP SDU received from upper layers, the transmitting PDCP entity shall:

-	associate the COUNT value corresponding to TX_NEXT to this PDCP SDU;

NOTE 1:	Associating more than half of the PDCP SN space of contiguous PDCP SDUs with PDCP SNs, when e.g., the PDCP SDUs are discarded or transmitted without acknowledgement, may cause HFN desynchronization problem. How to prevent HFN desynchronization problem is left up to UE implementation.

-	perform header compression of the PDCP SDU as specified in the subclause 5.7.4;

-	perform integrity protection, and ciphering using the TX_NEXT as specified in the subclause 5.9 and 5.8, respectively;

-	set the PDCP SN of the PDCP Data PDU to TX_NEXT modulo 2[pdcp-SN-Size];

-	increment TX_NEXT by one;

-	submit the resulting PDCP Data PDU to lower layer as specified below.

When submitting a PDCP PDU to lower layer, the transmitting PDCP entity shall:

-	if the transmitting PDCP entity is associated with one RLC entity:

-	submit the PDCP PDU to the associated RLC entity;

-	else, if the transmitting PDCP entity is associated with two RLC entities:

-	if pdcp-Duplication is configured and activated:

-	duplicate the PDCP Data PDU and submit the PDCP Data PDU to both associated RLC entities;

-	else, if pdcp-Duplication is configured but not activated:

-	submit the PDCP Data PDU to the primary RLC entity;

-	else:

-	if the total amount of PDCP data volume and RLC data volume pending for initial transmission (as specified in TS 38.322 [5]) in the two associated RLC entities is less than ul-DataSplitThreshold:

-	submit the PDCP PDU to the primary RLC entity;

-	else:

-	submit the PDCP PDU to either the primary RLC entity or the secondary RLC entity.

NOTE 2:	If the transmitting PDCP entity is associated with two RLC entities, the UE should minimize the amount of PDCP PDUs submitted to lower layers before receiving request from lower layers and minimize the PDCP SN gap between PDCP PDUs submitted to two associated RLC entities to minimize PDCP reordering delay in the receiving PDCP entity.

[TS 38.323, clause 5.4.1]

For AM DRBs configured by upper layers to send a PDCP status report in the uplink (statusReportRequired in TS 38.331 [3]), the receiving PDCP entity shall trigger a PDCP status report when:

-	upper layer requests a PDCP entity re-establishment;

-	upper layer requests a PDCP data recovery.

If a PDCP status report is triggered, the receiving PDCP entity shall:

-	compile a PDCP status report as indicated below by:

-	setting the FMC field to RX_DELIV;

-	if RX_DELIV < RX_NEXT:

-	allocating a Bitmap field of length in bits equal to the number of COUNTs from and not including the first missing PDCP SDU up to and including the last out-of-sequence PDCP SDUs, rounded up to the next multiple of 8, or up to and including a PDCP SDU for which the resulting PDCP Control PDU size is equal to 9000 bytes, whichever comes first;

-	setting in the bitmap field as '0' for all PDCP SDUs that have not been received, and optionally PDCP SDUs for which decompression have failed;

-	setting in the bitmap field as '1' for all PDCP SDUs that have been received;

-	submit the PDCP status report to lower layers as the first PDCP PDU for transmission.

[TS 38.323, clause 5.4.2]

For AM DRBs, when a PDCP status report is received in the downlink, the transmitting PDCP entity shall:

-	consider for each PDCP SDU, if any, with the bit in the bitmap set to '1', or with the associated COUNT value less than the value of FMC field as successfully delivered, and discard the PDCP SDU as specified in the subclause 5.3.

[TS 38.323, clause 5.5]

For AM DRBs, when upper layers request a PDCP data recovery for a radio bearer, the transmitting PDCP entity shall:

-	perform retransmission of all the PDCP Data PDUs previously submitted to re-established or released AM RLC entity in ascending order of the associated COUNT values for which the successful delivery has not been confirmed by lower layers.

After performing the above procedures, the transmitting PDCP entity shall follow the procedures in subclause 5.2.1.

7.1.3.5.3.3	Test description

7.1.3.5.3.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.3.0 except that DRB is configured in RLC AM mode according to Table 7.1.3.5.3.3.1-1.

For NR 5GC, NRCell 1 is the PCell and NR Cell 10 is the PSCell and same Pre-test conditions as in clause 7.1.3.0 using generic procedure parameter Connectivity (NR-DC).

Table 7.1.3.5.3.3.1-1: RLC parameters

| t-PollRetransmit | ms150 |
| --- | --- |

7.1.3.5.3.3.2	Test procedure sequence

Table 7.1.3.5.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Void | - | - | - | - |
| 2 | The SS creates 3 PDCP Data PDUs and the Next_PDCP_TX_SN is set to "0". | - | - | - | - |
| - | EXCEPTION: Steps 2A and 4 shall be repeated for k=0 to 2 (increment=1). | - | - | - | - |
| 2A | The SS is configured on PSCell to not send RLC acknowledgement (RLC ACK) to the UE. | - | - | - | - |
| 3 | The SS sends the PDCP Data PDU #k on SCG DRB on (PSCell): D/C field = 1 (PDCP Data PDU) and PDCP SN = k. After having sent a PDU, the SS sets Next_PDCP_TX_SN= k+1. | <-- | PDCP PDU DATA #k | - | - |
| 4 | The UE sends the PDCP Data PDU #k on the AM RLC entity configured for PSCell: D/C field = 1 (PDCP Data PDU) and PDCP SN = k. Data is previously received data from PDU #k. | --> | PDCP PDU DATA #k | - | - |
| 4A | The SS does not allocate any UL grant. | - | - | - | - |
| 5 | The SS transmits a NR RRCReconfiguration. (Note 1). | <-- | RRCReconfiguration | - | - |
| 6 | The SS assigns 1 UL grant of sufficient size. (Note 4) | - | - | - | - |
| - | EXCEPTION: Steps 7 and 8 can occur in any order. (Note 3) | - | - | - | - |
| 7 | The UE transmits a NR RRCReconfigurationComplete. (Note 2) | --> | RRCReconfigurationComplete | - | - |
| 8 | The UE sends PDCP Control PDUs via RLC-AM RB entity configured for PSCell Connectivity (EN-DC) and PCell for Connectivity (NR-DC)with the following content to the SS: D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 3. | --> | PDCP STATUS REPORT | - | - |
| 8A | After 100 ms the SS allocates 3 UL grants every 20ms of sufficient size to enable the UE to return each received PDCP PDU in one looped back PDCP PDU on PSCell for Connectivity (EN-DC) and PCell for Connectivity (NR-DC). | - | - | - | - |
| - | EXCEPTION: Step 9 shall be repeated for k=0 to 2 (increment=1). | - | - | - | - |
| 9 | Check: Does the UE send the PDCP Data PDU #k via the AM RLC entity configured for PSCell for Connectivity EN-DC and PCell for Connectivity (NR-DC). D/C field = 1 (PDCP Data PDU) and PDCP SN = k. Data is previously received data from PDU #k ? | --> | PDCP DATA PDU #k | 1 | P |
| Note 1: For EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration. Note 2: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. Note 3: Per 38.508-1 Table 4.6.3-66: LogicalChannelConfig, both SRB1 and DRB have the same logical channel priority with prioritisedBitRate as infinity. Note 4: For EN-DC, 104-bits of grant size is provided to include 10 bytes of PDCP Status report MAC PDU+ 3 bytes of Short BSR and/or Padding. For NR-DC, 272-bits of grant size to include 5 bytes of RLC STATUS MAC PDU, 10 bytes of PDCP Status report MAC PDU, 14 bytes of RRCReconfigurationComplete MAC PDU, 2 bytes of Short BSR and 3 bytes of Padding. |  |  |  |  |  |

7.1.3.5.3.3.3	Specific message contents

Table 7.1.3.5.3.3.3-1: RRCConnectionReconfiguration (step 5, Table 7.1.3.5.3.3.2-1)

| Derivation Path: TS 36.508 [7], Table 4.6.1-8 with condition MCG_and_SCG |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nr-Config-r15 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| nr-SecondaryCellGroupConfig-r15 | OCTET STRING including the RRCReconfiguration-PDCP message and the IE secondaryCellGroup |  |  |
| } |  |  |  |
| } |  |  |  |
| nr-RadioBearerConfig1-r15 | OCTET STRING including RadioBearerConfig-PDCP-ENDC |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.5.3.3.3-2: RRCReconfiguration-PDCP (Table 7.1.3.5.3.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | Not present |  | NR-DC |
| secondaryCellGroup | CellGroupConfig-PDCP-ENDC |  | EN-DC |
|  | Not present |  | NR-DC |
| nonCriticalExtension SEQUENCE { |  |  | NR-DC |
| masterCellGroup | CellGroupConfig with condition DRBn as per 38.508-1[4] Table 4.6.3-19 | SCG DRBn added in master cell group |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| mrdc-SecondaryCellGroupConfig CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| mrdc-SecondaryCellGroup CHOICE { |  |  |  |
| nr-SCG | RRCReconfiguration-SCGRLCreEst | OCTET STRING (CONTAINING RRCReconfiguration) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| radioBearerConfig2 | RadioBearerConfig-PDCPsplit |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.5.3.3.3-2A: RRCReconfiguration-SCGRLCreEst (Table 7.1.3.5.3.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition NR-DC_SCG |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| secondaryCellGroup |  | CellGroupConfig-SCG-RLC | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.3.5.3.3.3-2B: CellGroupConfig-PDCP-ENDC (Table 7.1.3.5.3.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 1 entry |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions AM and DRB2 and Re-establish_RLC | entry 1 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.5.3.3.3-2C: CellGroupConfig-SCG-RLC (Table 7.1.3.5.3.3.3-2A)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with condition NR-DC_SCG |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 1 entry |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions AM and DRBn and Re-establish_RLC |  |  |
| } |  |  |  |
| mac-CellGroupConfig | Not present |  |  |
| physicalCellGroupConfig | Not present |  |  |
| spCellConfig | Not present |  |  |
| } |  |  |  |

Table 7.1.3.5.3.3.3-3: RadioBearerConfig-PDCP-ENDC (Table 7.1.3.5.3.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| srb3-ToRelease | Not present |  |  |
| drb-ToAddModList | Not present |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| eps-BearerIdentity | 6 |  |  |
| sdap-Config | Not present |  |  |
| } |  |  |  |
| drb-Identity | 2 |  |  |
| reestablishPDCP | Not present |  |  |
| recoverPDCP | true |  |  |
| pdcp-Config | PDCP-Config-Split |  |  |
| } |  |  |  |
| } |  |  |  |
| drb-ToReleaseList | Not present |  |  |
| } |  |  |  |

Table 7.1.3.5.3.3.3-3A: RadioBearerConfig-PDCPsplit (Table 7.1.3.5.3.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  |  |  |
| cnAssociation | Not present |  |  |
| drb-Identity | DRB-Identity with condition DRBn | SCG DRBn |  |
| reestablishPDCP | Not present |  |  |
| recoverPDCP | true |  |  |
| pdcp-Config | PDCP-Config-Split |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.5.3.3.3-4: PDCP-Config-Split (Table 7.1.3.5.3.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-99 with condition Split |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-Config ::= SEQUENCE { |  |  |  |
| moreThanOneRLC SEQUENCE { |  |  |  |
| primaryPath SEQUENCE { |  |  |  |
| cellGroup | 1 |  | EN-DC |
|  | 0 | MCG Path | NR-DC |
| } |  |  |  |
| ul-DataSplitThreshold | infinity |  |  |
| } |  |  |  |
| } |  |  |  |

##### 7.1.3.5.4	PDCP reordering / Maximum re-ordering delay below t-Reordering / t-Reordering timer operations

7.1.3.5.4.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state using RLC-AM }

ensure that {

when { a PDCP PDU is received from the lower layers and the COUNT value of the received PDCP Data PDU is out of the re-ordering window }

then { UE discards the PDCP PDU }

}

(2)

with { UE in RRC_CONNECTED state using RLC-AM }

ensure that {

when { a PDCP PDU is received from the lower layers and the COUNT value of the received PDCP Data PDU is within the re-ordering window }

then { UE stores the resulting PDCP SDU }

}

(3)

with { UE in RRC_CONNECTED state using RLC-AM, and the RX_DELIV is not equal to the COUNT value of the RX_NEXT (there is missing PDCP PDUs) }

ensure that {

when { a PDCP PDU is received from the lower layers and the RCVD_COUNT = RX_DELIV }

then { UE delivers the resulting PDCP SDU and all stored PDCP SDUs with consecutive COUNT value to upper layer, in ascending order }

}

(4)

with { UE in RRC_CONNECTED state using RLC-AM and the associated PDCP t-Reordering timer is running }

ensure that {

when { RX_DELIV >= RX_REORD }

then { UE stops and resets t-Reordering timer }

}

(5)

with { UE in RRC_CONNECTED state using RLC-AM and the associated PDCP t-Reordering timer is running }

ensure that {

when { the t-Reordering timer expires }

then { UE delivers all stored PDCP SDUs to upper layer }

}

(6)

with { UE in RRC_CONNECTED state using RLC-AM and the associated PDCP t-Reordering timer is running }

ensure that {

when { the t-Reordering is reconfigured by upper layers }

then { UE stops and resets t-Reordering timer }

}

7.1.3.5.4.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.323, clause 5.2.2.1, 5.2.2.2 and 5.2.2.3. Unless otherwise stated these are Rel-15 requirements.

[TS 38.323, clause 5.2.2.1]

In this section, following definitions are used:

-	HFN(State Variable): the HFN part (i.e. the number of most significant bits equal to HFN length) of the State Variable;

-	SN(State Variable): the SN part (i.e. the number of least significant bits equal to PDCP SN length) of the State Variable;

-	RCVD_SN: the PDCP SN of the received PDCP Data PDU, included in the PDU header;

-	RCVD_HFN: the HFN of the received PDCP Data PDU, calculated by the receiving PDCP entity;

-	RCVD_COUNT: the COUNT of the received PDCP Data PDU = [RCVD_HFN, RCVD_SN].

At reception of a PDCP Data PDU from lower layers, the receiving PDCP entity shall determine the COUNT value of the received PDCP Data PDU, i.e. RCVD_COUNT, as follows:

-	if RCVD_SN < SN(RX_DELIV) – Window_Size:

-	RCVD_HFN = HFN(RX_DELIV) + 1.

-	else if RCVD_SN >= SN(RX_DELIV) + Window_Size:

-	RCVD_HFN = HFN(RX_DELIV) – 1.

-	else:

-	RCVD_HFN = HFN(RX_DELIV);

-	RCVD_COUNT = [RCVD_HFN, RCVD_SN].

After determining the COUNT value of the received PDCP Data PDU = RCVD_COUNT, the receiving PDCP entity shall:

-	perform deciphering and integrity verification of the PDCP Data PDU using COUNT = RCVD_COUNT;

-	if integrity verification fails:

-	indicate the integrity verification failure to upper layer;

-	discard the PDCP Data PDU;

-	if RCVD_COUNT < RX_DELIV; or

-	if the PDCP Data PDU with COUNT = RCVD_COUNT has been received before:

-	discard the PDCP Data PDU;

If the received PDCP Data PDU with COUNT value = RCVD_COUNT is not discarded above, the receiving PDCP entity shall:

-	store the resulting PDCP SDU in the reception buffer;

-	if RCVD_COUNT >= RX_NEXT:

-	update RX_NEXT to RCVD_COUNT + 1.

-	if outOfOrderDelivery is configured:

-	deliver the resulting PDCP SDU to upper layers.

-	if RCVD_COUNT = RX_DELIV:

-	deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before;

-	all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from COUNT = RX_DELIV;

-	update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers, with COUNT value > RX_DELIV;

-	if t-Reordering is running, and if RX_DELIV >= RX_REORD:

-	stop and reset t-Reordering.

-	if t-Reordering is not running (includes the case when t-Reordering is stopped due to actions above), and RX_DELIV < RX_NEXT:

-	update RX_REORD to RX_NEXT;

-	start t-Reordering.

[TS 38.323, clause 5.2.2.2]

When t-Reordering expires, the receiving PDCP entity shall:

-	deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before:

-	all stored PDCP SDU(s) with associated COUNT value(s) < RX_REORD;

-	all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from RX_REORD;

-	update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers, with COUNT value >= RX_REORD;

-	if RX_DELIV < RX_NEXT:

-	update RX_REORD to RX_NEXT;

-	start t-Reordering.

[TS 38.323, clause 5.2.2.3]

When the value of the t-Reordering is reconfigured by upper layers while the t-Reordering is running, the receiving PDCP entity shall:

-	update RX_REORD to RX_NEXT;

-	stop and restart t-Reordering.

7.1.3.5.4.3	Test description

7.1.3.5.4.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.3.0 exception  of PDCP parameters according to Table 7.1.3.5.4.3.1-1.

Table 7.1.3.5.4.3.1-1: PDCP parameters

| t-Reordering | ms300 |
| --- | --- |

7.1.3.5.4.3.2	Test procedure sequence

Table 7.1.3.5.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS sends the PDCP SDU #X D/C field = 1 (PDCP Data PDU) and PDCP SN = X (Note 1) (Note 6) | <-- | (PDCP SDU #X) | - | - |
| 2 | Check: Does the UE transmit a PDCP SDU via the AM RLC entity in the next 1s? | --> | (PDCP SDU) | 1 | F |
| 3 | The SS sends the PDCP SDU #1 D/C field = 1 (PDCP Data PDU) and PDCP SN = 1. The UE starts t-Reordering. | <-- | (PDCP SDU #1) | - | - |
| 4 | The SS sends the PDCP SDU #2 D/C field = 1 (PDCP Data PDU) and PDCP SN =2. | <-- | (PDCP SDU #2) | - | - |
| 5 | Wait for 100ms (< configured t-Reordering). | - | - | - | - |
| 6 | The SS sends the PDCP SDU #0 D/C field = 1 (PDCP Data PDU) and PDCP SN = 0. | <-- | (PDCP SDU #0) | - | - |
| 7 | Check: Does the UE transmit the PDCP SDU #0 via the AM RLC entity D/C field = 1 (PDCP Data PDU) and PDCP SN = 0? (Note 4) | --> | (PDCP SDU #0) | 2, 3 | P |
| 8 | Check: Does the UE transmit the PDCP SDU #1 via the AM RLC entity D/C field = 1 (PDCP Data PDU) and PDCP SN = 1? (Note 4) | --> | (PDCP SDU #1) | 2, 3 | P |
| 9 | Check: Does the UE transmit the PDCP SDU #2 via the AM RLC entity D/C field = 1 (PDCP Data PDU) and PDCP SN = 2? (Note 4) | --> | (PDCP SDU #2) | 2, 3 | P |
| 10 | The SS sends the PDCP SDU #4 D/C field = 1 (PDCP Data PDU) and PDCP SN = 4. The UE starts t-Reordering. | <-- | (PDCP SDU #4) | - | - |
| 11 | Wait for 100ms (< configured t-Reordering) | - | - | - | - |
| 12 | The SS sends the PDCP SDU #7 D/C field = 1 (PDCP Data PDU) and PDCP SN = 7. | <-- | (PDCP SDU #7) | - | - |
| 13 | The SS sends the PDCP SDU #3 D/C field = 1 (PDCP Data PDU) and PDCP SN = 3. The UE restarts t-Reordering timer. Note T1 | <-- | (PDCP SDU #3) | - | - |
| 14 | Check: Does the UE transmit the PDCP SDU #3 via the AM RLC entity? (Note 5) | --> | (PDCP SDU #3) | 3 | P |
| 15 | Check: Does the UE transmit the PDCP SDU #4 via the AM RLC entity? (Note 5) | --> | (PDCP SDU #4) | 3 | P |
| 16 | Check 1: Does the UE transmit the PDCP SDU #7 with PDCP SN=5 via the AM RLC entity after t-Reordering expiry? Note T2 Check 2: Is (T2 – T1) > t-Reordering? | --> | (PDCP SDU #7) | 4,5 | P |
| 17 | The SS sends the PDCP SDU #9 D/C field = 1 (PDCP Data PDU) and PDCP SN = 9. The UE starts t-Reordering. | <-- | (PDCP SDU #9) | - | - |
| 18 | Wait for 100ms (< configured t-Reordering) | - | - | - | - |
| 19 | The SS reconfigures the t-Reordering by sending a NR RRCReconfiguration message. The UE restarts t-Reordering timer. (Note 2) Note T3 | <-- | (RRCReconfiguration) | - | - |
| 20 | The UE transmits a NR RRCReconfigurationComplete message. (Note 3) | --> | (RRCReconfigurationComplete) | - | - |
| 21 | Check 1: Does the UE transmit the PDCP SDU #9 with PDCP SN=6 via the AM RLC entity after t-Reordering expiry? Note T4 Check 2: Is (T4 – T3) > t-Reordering? | --> | (PDCP SDU #9) | 6 | P |
| Note 1: The Reordering Window size is 131072 when 18 bit SN length is used. Note 2: For EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration 36.508 [7], Table 4.6.1-8 using condition EN-DC_Embed_RBConfig. Note 3: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. Note 4: PDCP SDUs at steps 7, 8 and 9 may be received by the SS in the same slot or in multiple slots (max one MAC PDU in a slot) Note 5: PDCP SDUs at steps 14 and 15 may be received by the SS in the same slot or in multiple slots (max one MAC PDU in a slot). Note 6: X=131072 for (pc_supportOfRedCap_r17=false AND pc_supportOfERedCap_r18= false) and X=2048 for (pc_supportOfRedCap_r17=true OR pc_supportOfERedCap_r18=true). |  |  |  |  |  |

7.1.3.5.4.3.3	Specific message contents

Table 7.1.3.5.4.3.3-1: RRCReconfiguration (step 19, Table 7.1.3.5.4.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table: 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig |  | Not EN-DC |
| secondaryCellGroup | CellGroupConfig |  | EN-DC |
| nonCriticalExtension SEQUENCE { |  |  | Not EN-DC |
| masterCellGroup | CellGroupConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.5.4.3.3-2: RadioBearerConfig (Table 7.1.3.5.4.3.3-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList ::= SEQUENCE (SIZE 1..2)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| pdcp-Config SEQUENCE { |  |  |  |
| drb SEQUENCE { |  |  |  |
| outOfOrderDelivery | Not present |  |  |
| } |  |  |  |
| t-Reordering | ms750 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 7.1.3.5.5	PDCP Duplication

7.1.3.5.5.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state  and pdcpDuplication is configured and activated}

ensure that {

when { UE has PDCP SDUs available for transmission}

then { the UE transmits the PDCP SDUs on  both the associated RLC entities }

}

(2)

with { UE in RRC_CONNECTED state and pdcpDuplication is configured and not activated }

ensure that {

when { UE receives MAC Control Element to Activate PDCP Duplication on a DRB configured with PDCP duplication }

then { the UE activates PDCP Duplication on the PDCP associated with the DRB }

}

(3)

with { UE in RRC_CONNECTED state and pdcpDuplication is configured }

ensure that {

when { UE has PDCP SDUs available for transmission }

then { the UE transmits the PDCP SDUs on the primary RLC entity}

}

(4)

with { UE in RRC_CONNECTED state  and pdcpDuplication is configured and activated}

ensure that {

when { the UE had transmited the PDCP SDUs on  both the associated RLC entities and successful delivery of a PDCP Data PDU is confirmed by one of the two associated AM RLC entities}

then { the other AM RLC entity discards the duplicated PDCP Data PDU}

}

7.1.3.5.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in:  TS 38.321:5.10, 6.1.3.10 and TS 38.323:5.2.1, 5.11.1, 5.11.2 and TS 38.331: 5.3.5.6.4. Unless otherwise stated these are Rel-15 requirements.

[TS 38.323, clause 5.2.1]

When submitting a PDCP PDU to lower layer, the transmitting PDCP entity shall:

-	if the transmitting PDCP entity is associated with one RLC entity:

-	submit the PDCP PDU to the associated RLC entity;

-	else, if the transmitting PDCP entity is associated with two RLC entities:

-	if the PDCP duplication is activated:

-	if the PDCP PDU is a PDCP Data PDU:

-	duplicate the PDCP Data PDU and submit the PDCP Data PDU to both associated RLC entities;

-	else:

-	submit the PDCP Control PDU to the primary RLC entity;

-	else:

-	if the two associated RLC entities belong to the different Cell Groups; and

-	if the total amount of PDCP data volume and RLC data volume pending for initial transmission (as specified in TS 38.322 [5]) in the two associated RLC entities is equal to or larger than ul-DataSplitThreshold:

-	submit the PDCP PDU to either the primary RLC entity or the secondary RLC entity;

-	else:

-	submit the PDCP PDU to the primary RLC entity.

[TS 38.331, clause 5.3.5.6.4]

The UE shall:

1>	for each drb-Identity value included in the drb-ToReleaseList that is part of the current UE configuration; or

1>	for each drb-Identity value that is to be released as the result of full configuration according to 5.3.5.11:

2>	release the PDCP entity and the drb-Identity;

2>	if SDAP entity associated with this DRB is configured:

3>	indicate the release of the DRB to SDAP entity associated with this DRB (TS 37.324 [24], clause 5.3.3);

2>	if the UE is operating in EN-DC:

3>	if a new bearer is not added either with NR or E-UTRA with same eps-BearerIdentity:

4>	indicate the release of the DRB and the eps-BearerIdentity of the released DRB to upper layers.

NOTE 1:	The UE does not consider the message as erroneous if the drb-ToReleaseList includes any drb-Identity value that is not part of the current UE configuration.

NOTE 2:	Whether or not the RLC and MAC entities associated with this PDCP entity are reset or released is determined by the CellGroupConfig.

[TS 38.323, clause 5.11.1]

For the PDCP entity configured with pdcp-Duplication, the transmitting PDCP entity shall:

-	for SRBs:

-	activate the PDCP duplication;

-	for DRBs:

-	if the activation of PDCP duplication is indicated:

-	activate the PDCP duplication;

-	if the deactivation of PDCP duplication is indicated:

-	deactivate the PDCP duplication.

[TS 38.323, clause 5.11.2]

For the PDCP entity configured with pdcp-Duplication, the transmitting PDCP entity shall:

-	if the successful delivery of a PDCP Data PDU is confirmed by one of the two associated AM RLC entities:

-	indicate to the other AM RLC entity to discard the duplicated PDCP Data PDU;

-	if the deactivation of PDCP duplication is indicated:

-	indicate to the secondary RLC entity to discard all duplicated PDCP Data PDUs.

[TS 38.321, clause 5.10]

If one or more DRBs are configured with PDCP duplication, the network may activate and deactivate the PDCP duplication for the configured DRB(s).

The PDCP duplication for the configured DRB(s) is activated and deactivated by:

-	receiving the Duplication Activation/Deactivation MAC CE described in subclause 6.1.3.11;

-	indication by RRC.

The MAC entity shall for each DRB configured with PDCP duplication:

1>	if a Duplication Activation/Deactivation MAC CE is received activating the PDCP duplication of the DRB:

2>	indicate the activation of PDCP duplication of the DRB to upper layers.

1>	if a Duplication Activation/Deactivation MAC CE is received deactivating the PDCP duplication of the DRB:

2>	indicate the deactivation of PDCP duplication of the DRB to upper layers.

7.1.3.5.5.3	Test description

7.1.3.5.5.3.1	Pre-test conditions

System Simulator:

-	For NR 5GC NR Cell 1 and NR Cell 10

UE:

-	None

Preamble:

-	Same Pre-test conditions as in clause 7.1.3.0 and Generic procedure parameter DC bearer (MCG and split).

-	For NR 5GC, Same Pre-test conditions as in clause 7.1.3.0 using generic procedure parameter Connectivity (NR-DC), Bearers(MCG(s) and Split),

7.1.3.5.5.3.2	Test procedure sequence

Table 7.1.3.5.5.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Void | - | - | - | - |
| 2 | SS transmits a Duplication Activation MAC CE to activate PDCP Duplication for split DRB. | <-- | MAC PDU (Duplication Activation MAC Control Element) | - | - |
| 2A | The SS is configured not to send RLC ACK for the next PDU on split DRB. (Note 6) | - | - | - | - |
| 3 | The SS sends a PDCP Data PDU on the split DRB on the AM RLC primary entity. | <-- | PDCP DATA PDU | - | - |
| - | EXCEPTION: Steps 4-4A below occur in any sequence | - | - | - | - |
| 4 | Check: Does UE transmit a PDCP Data PDU on the AM RLC primary entity? | --> | PDCP DATA PDU | 3 | P |
| 4A | Check: Does UE transmit a PDCP Data PDU on the AM RLC secondary entity? | --> | PDCP DATA PDU | 2 | P |
| 4B-4C | Void | - | - | - | - |
| 5 | SS transmits a Duplication Deactivation MAC CE to deactivate PDCP Duplication for split DRB. | <-- | MAC PDU (Duplication Deactivation MAC Control Element) | - | - |
| 6 | The SS sends a PDCP Data PDU on the split DRB on the AM RLC primary entity. | <-- | PDCP DATA PDU | - | - |
| 7 | Check: Does UE transmit a PDCP Data PDU on the AM RLC entity configured on the AM RLC primary entity? | --> | PDCP DATA PDU | 3 | P |
| 8 | The SS transmits an NR RRCReconfiguration message to activate parameters for PdcpDuplication(Note 2) | <-- | RRCReconfiguration | - | - |
| 8A | UE responses NR RRCReconfigurationComplete message.(Note 3) | --> | RRCReconfigurationComplete | - | - |
| 8AA | The SS is configured not to send RLC ACK for the next PDU on split DRB. (Note 6) | - | - | - | - |
| 8B | The SS sends a PDCP Data PDU on the split DRB on the AM RLC primary entity. | <-- | PDCP DATA PDU | - | - |
| - | EXCEPTION: Steps 8C-8D below occur in any sequence | - | - | - | - |
| 8C | Check: Does UE transmit a PDCP Data PDU on the AM RLC primary entity? | --> | PDCP DATA PDU | 1 | P |
| 8D | Check: Does UE transmit a PDCP Data PDU on the AM RLC secondary entity? | --> | PDCP DATA PDU | 1 | P |
| 8E-8F | Void | - | - | - | - |
| 9 | The SS stops allocating any UL grant for the AM RLC secondary entity(Note 1) | - | - | - | - |
| 10 | The SS sends a PDCP Data PDU on the split DRB on the AM RLC primary entity. | <-- | PDCP DATA PDU | - | - |
| 11 | UE transmits a PDCP Data PDU on the AM RLC primary entity | --> | PDCP DATA PDU | - | - |
| 12 | The SS resumes normal UL grant allocation for the AM RLC secondary entity. | - | - | - | - |
| 13 | Check: Does UE transmit a PDCP Data PDU on the AM RLC primary or secondary entity in next five seconds? | --> | PDCP DATA PDU | 4 | F |
| Note 1: Discard of RLC SDU is not possible if submitted to lower layers. Therefore, Grant is not provided so that RLC SDU is not submitted to lower layers. Note 2: For EN-DC the NR RRCReconfiguration is contained in RRCConnectionReconfiguration Table 7.1.3.5.5.3.3-4. Note 3: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. Note 4: Void Note 5: Void Note 6: SS is configured not to send RLC ACK for the next PDU on split DRB to ensure that UE side primary or secondary entities do not discard the PDU before PDSCH transmission upon receipt of RLC ACK on one of the entities. This may cause UE side entities to retransmit the PDU but it shall be ignored by SS side RLC entity as it has already been received though not acknowledged due to SS side RLC being configured in test mode. |  |  |  |  |  |

7.1.3.5.5.3.3	Specific message contents

Table 7.1.3.5.5.3.3-0A: RRCConnectionReconfiguration (step 8, Table 7.1.3.5.5.3.2-1)

| Derivation Path: 36.508 [7], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nr-RadioBearerConfig1-r15 | OCTET STRING including RadioBearerConfig-PDCP |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.5.5.3.3-0B: RRCReconfiguration (step 8, Table 7.1.3.5.5.3.2-1)

| Derivation Path: 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| radioBearerConfig2 | OCTET STRING including RadioBearerConfig-PDCP |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.5.5.3.3-1: RadioBearerConfig-PDCP (Table 7.1.3.5.5.3.3-0A and Table 7.1.3.5.5.3.3-0B , Step 8)

| Derivation Path: TS 38.508-1 [7], Table 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| srb3-ToRelease | Not present |  |  |
| drb-ToAddModList | Not present |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| drb-Identity | 2 |  | EN-DC |
|  | SCG DRBn |  | NR |
| pdcp-Config | PDCP-Config-Split |  |  |
| } |  |  |  |
| } |  |  |  |
| drb-ToReleaseList | Not present |  |  |
| } |  |  |  |

Table 7.1.3.5.5.3.3-2: PDCP-Config-Split (Table 7.1.3.5.5.3.3-1)

| Derivation Path: 38.508-1 [4], Table 4.6.3-99 condition Split |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-Config ::= SEQUENCE { |  |  |  |
| moreThanOneRLC SEQUENCE { |  |  |  |
| primaryPath SEQUENCE { |  |  |  |
| cellGroup | 1 |  | EN-DC |
|  | 0 |  | NR |
| } |  |  |  |
| ul-DataSplitThreshold | infinity |  |  |
| pdcp-Duplication | true |  |  |
| } |  |  |  |
| } |  |  |  |

##### 7.1.3.5.6	PDCP Duplication / 3 RLC entities

###### 7.1.3.5.6.1	PDCP Duplication / 3 RLC entities / Intra-band Contiguous CA

7.1.3.5.6.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and PDCP CA duplication is configured and activated }

ensure that {

when { UE has a PDCP SDU available for transmission }

then { the UE transmits the PDCP SDU on all the associated RLC entities }

}

(2)

Void

(3)

with { UE in RRC_CONNECTED state and PDCP CA duplication is configured }

ensure that {

when { UE has a PDCP SDU available for transmission }

then { the UE transmits the PDCP SDU on the primary RLC entity }

}

(4)

with { UE in RRC_CONNECTED state and PDCP CA duplication is configured and activated }

ensure that {

when { the UE had transmitted the PDCP SDU on all the associated RLC entities and successful delivery of a PDCP Data PDU is confirmed by one of the associated AM RLC entities }

then { the other AM RLC entities discard the duplicated PDCP Data PDU }

}

(5)

with { UE in RRC_CONNECTED state and PDCP CA duplication is configured and activated }

ensure that {

when { UE receives MAC Control Element to deactivate PDCP Duplication on one of the associated RLC entity configured with PDCP duplication }

then { the UE deactivates the PDCP duplication for the indicated associated RLC entity }

7.1.3.5.6.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in:  TS 38.321:5.10, 6.1.3.11, 6.1.3.32, TS 38.323:5.2.1, 5.11.1, 5.11.2 and TS 38.331: 5.3.5.6.4. Unless otherwise stated these are Rel-16 requirements.

[TS 38.323, clause 5.2.1]

When submitting a PDCP PDU to lower layer, the transmitting PDCP entity shall:

-	if the transmitting PDCP entity is associated with one RLC entity:

-	submit the PDCP PDU to the associated RLC entity;

-	else, if the transmitting PDCP entity is associated with at least two RLC entities:

-	if the PDCP duplication is activated for the RB:

-	if the PDCP PDU is a PDCP Data PDU:

-	duplicate the PDCP Data PDU and submit the PDCP Data PDU to the associated RLC entities activated for PDCP duplication;

-	else:

-	submit the PDCP Control PDU to the primary RLC entity;

-	else (i.e. the PDCP duplication is deactivated for the RB):

-	if the split secondary RLC entity is configured; and

-	if the total amount of PDCP data volume and RLC data volume pending for initial transmission (as specified in TS 38.322 [5]) in the primary RLC entity and the split secondary RLC entity is equal to or larger than ul-DataSplitThreshold:

-	submit the PDCP PDU to either the primary RLC entity or the split secondary RLC entity;

-	else, if the transmitting PDCP entity is associated with the DAPS bearer:

-	if the uplink data switching has not been requested:

-	submit the PDCP PDU to the RLC entity associated with the source cell;

-	else:

-	if the PDCP PDU is a PDCP Data PDU:

-	submit the PDCP Data PDU to the RLC entity associated with the target cell;

-	else:

-	if the PDCP Control PDU is associated with source cell:

-	submit the PDCP Control PDU to the RLC entity associated with the source cell;

-	else:

-	submit the PDCP Control PDU to the RLC entity associated with the target cell;

-	else:

-	submit the PDCP PDU to the primary RLC entity.

[TS 38.323, clause 5.11.1]

For the PDCP entity configured with pdcp-Duplication, the transmitting PDCP entity shall:

-	for SRBs:

-	activate the PDCP duplication;

-	for DRBs:

-	if the activation of PDCP duplication is indicated for the DRB:

-	activate the PDCP duplication for the DRB;

-	if the activation of PDCP duplication is indicated for at least one associated RLC entities:

-	activate the PDCP duplication for the indicated associated RLC entities;

-	activate the PDCP duplication for the DRB;

-	if the deactivation of PDCP duplication is indicated for the DRB:

-	deactivate the PDCP duplication for the DRB;

-	if the deactivation of PDCP duplication is indicated for at least one associated RLC entities:

-	deactivate the PDCP duplication for the indicated associated RLC entities;

-	if all associated RLC entities other than the primary RLC entity are deactivated for PDCP duplication:

-	deactivate the PDCP duplication for the DRB.

[TS 38.323, clause 5.11.2]

For the PDCP entity configured with pdcp-Duplication, the transmitting PDCP entity shall:

-	if the successful delivery of a PDCP Data PDU is confirmed by one of the associated AM RLC entities:

-	indicate to the other AM RLC entities to discard the duplicated PDCP Data PDU;

-	if the deactivation of PDCP duplication is indicated for the DRB:

-	indicate to the RLC entities other than the primary RLC entity to discard all duplicated PDCP Data PDUs;

-	if the deactivation of PDCP duplication is indicated for at least one associated RLC entities:

-	indicate to the RLC entities deactivated for PDCP duplication to discard all duplicated PDCP Data PDUs.

[TS 38.321, clause 5.10]

If one or more DRBs are configured with PDCP duplication, the network may activate and deactivate the PDCP duplication for all or a subset of associated RLC entities for the configured DRB(s).

The PDCP duplication for the configured DRB(s) is activated and deactivated by:

-	receiving the Duplication Activation/Deactivation MAC CE described in clause 6.1.3.11;

-	receiving the Duplication RLC Activation/Deactivation MAC CE described in clause 6.1.3.32;

-	indication by RRC.

The PDCP duplication for all or a subset of associated RLC entities for the configured DRB(s) is activated and deactivated by:

-	receiving the Duplication RLC Activation/Deactivation MAC CE described in clause 6.1.3.32;

-	indication by RRC.

The MAC entity shall for each DRB configured with PDCP duplication:

1>	if a Duplication Activation/Deactivation MAC CE is received activating the PDCP duplication of the DRB:

2>	indicate the activation of PDCP duplication of the DRB to upper layers.

1>	if a Duplication Activation/Deactivation MAC CE is received deactivating the PDCP duplication of the DRB:

2>	indicate the deactivation of PDCP duplication of the DRB to upper layers.

1>	if a Duplication RLC Activation/Deactivation MAC CE is received activating PDCP duplication for associated RLC entities of a DRB configured with PDCP duplication:

2>	indicate the activation of PDCP duplication for the indicated secondary RLC entity(ies) of the DRB to upper layers.

1>	if a Duplication RLC Activation/Deactivation MAC CE is received deactivating PDCP duplication for associated RLC entities of a DRB configured with PDCP duplication:

2>	indicate the deactivation of PDCP duplication for the indicated secondary RLC entity(ies) of the DRB to upper layers.

[TS 38.331, clause 5.3.5.6.4]

The UE shall:

1>	for each drb-Identity value included in the drb-ToReleaseList that is part of the current UE configuration; or

1>	for each drb-Identity value that is to be released as the result of full configuration according to 5.3.5.11:

2>	release the PDCP entity and the drb-Identity;

2>	if SDAP entity associated with this DRB is configured:

3>	indicate the release of the DRB to SDAP entity associated with this DRB (TS 37.324 [24], clause 5.3.3);

2>	if the UE is operating in EN-DC:

3>	if a new bearer is not added either with NR or E-UTRA with same eps-BearerIdentity:

4>	indicate the release of the DRB and the eps-BearerIdentity of the released DRB to upper layers.

NOTE 1:	The UE does not consider the message as erroneous if the drb-ToReleaseList includes any drb-Identity value that is not part of the current UE configuration.

NOTE 2:	Whether or not the RLC and MAC entities associated with this PDCP entity are reset or released is determined by the CellGroupConfig.

7.1.3.5.6.1.3	Test description

7.1.3.5.6.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the PCell, NR Cell 3 and NR Cell 6 are the SCells.

UE:

-	None

Preamble:

-	Same Pre-test conditions as in clause 7.1.3.0.

7.1.3.5.6.1.3.2	Test procedure sequence

Table 7.1.3.5.6.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an NR RRCReconfiguration message to add 2 SCells and to configure parameters for PDCP CA duplication | <-- | RRCReconfiguration | - | - |
| 1A | UE transmits NR RRCReconfigurationComplete message. | --> | RRCReconfigurationComplete | - | - |
| 1B | The SS transmits a SCell Activation MAC-CE on PCell (NR Cell 1) to activate NR SCells (NR Cell 3 and NR Cell 6). | <-- | MAC PDU (SCell Activation/Deactivation MAC CE of one octet (C1=1)) | - | - |
| 2 | The SS sends a PDCP Data PDU on the DRB on the AM RLC primary entity. | <-- | PDCP DATA PDU | - | - |
| 3 | Check: Does UE transmit a PDCP Data PDU on the AM RLC primary entity? | --> | PDCP DATA PDU | 3 | P |
| 3A | SS transmits a MAC CE to activate PDCP Duplication for all AM RLC entities associated to the DRB. | <-- | MAC PDU (Duplication RLC Activation MAC Control Element) | - | - |
| 3B | The SS is configured not to send RLC ACK on all 3 RLC entities. (Note 2) | - | - | - | - |
| 3C | The SS sends a PDCP Data PDU on the DRB on the AM RLC primary entity. | <-- | PDCP DATA PDU | - | - |
|  | EXCEPTION: Steps 3D, 4 & 4A below can occur in any order. | - | - | - | - |
| 3D | Check: Does UE transmit a PDCP Data PDU on the AM RLC primary entity? | --> | PDCP DATA PDU | 1 | P |
| 4 | Check: Does UE transmit a PDCP Data PDU on the associated AM RLC entity on the SCell (NR Cell 3)? | --> | PDCP DATA PDU | 1 | P |
| 4A | Check: Does UE transmit a PDCP Data PDU on the associated AM RLC entity on the SCell (NR Cell 6)? | --> | PDCP DATA PDU | 1 | P |
| 5 | The SS transmits a MAC CE to deactivate PDCP Duplication for the AM RLC entity associated with the DRB on SCell (NR Cell 6). | <-- | MAC PDU (Duplication RLC Deactivation MAC Control Element) | - | - |
| 5A | The SS is configured not to send RLC ACK on all 3 RLC entities. | - | - | - | - |
| 6 | The SS sends a PDCP Data PDU on the DRB on the AM RLC primary entity. | <-- | PDCP DATA PDU | - | - |
| - | EXCEPTION: Steps 6A-6B below can occur in any order. | - | - | - | - |
| 6A | The UE transmits a PDCP Data PDU on the activated primary AM RLC entity. | --> | PDCP DATA PDU | - | - |
| 6B | The UE transmits a PDCP Data PDU on the activated secondary AM RLC entity. | --> | PDCP DATA PDU | - | - |
| 7 | Check: Does UE transmit a PDCP Data PDU on the deactivated AM RLC entity by step 5? | --> | PDCP DATA PDU | 5 | F |
| 8-14 | Void | - | - | - | - |
| 15 | The SS stops allocating any UL grant on the SCells.(Note 1) | - | - | - | - |
| 16 | The SS sends a PDCP Data PDU on the DRB on the AM RLC primary entity. | <-- | PDCP DATA PDU | - | - |
| 17 | UE transmits a PDCP Data PDU on the AM RLC primary entity | --> | PDCP DATA PDU | - | - |
| 18 | The SS resumes normal UL grant allocation on the SCells. | - | - | - | - |
| - | EXCEPTION: Steps 19-21 below can occur in any order in the next five seconds. | - | - | - | - |
| 19 | Check: Does UE transmit a PDCP Data PDU on the AM RLC primary? | --> | PDCP DATA PDU | 4 | F |
| 20 | Check: Does UE transmit a PDCP Data PDU on the associated AM RLC entity on the SCell (NR Cell 3)? | --> | PDCP DATA PDU | 4 | F |
| 21 | Check: Does UE transmit a PDCP Data PDU on the associated AM RLC entity on the SCell (NR Cell 6)? | --> | PDCP DATA PDU | 4 | F |
| Note 1: Discard of RLC SDU is not possible if submitted to lower layers. Therefore, Grant is not provided so that RLC SDU is not submitted to lower layers. Note 2: SS is configured not to send RLC ACK for the next PDU on all RLC entities to ensure that UE side primary or secondary entities do not discard the PDU before PDSCH transmission upon receipt of RLC ACK on one of the entities. This may cause UE side entities to retransmit the PDU but it shall be ignored by SS side RLC entity as it has already been received though not acknowledged due to SS side RLC being configured in test mode. |  |  |  |  |  |

7.1.3.5.6.1.3.3	Specific message contents

Table 7.1.3.5.6.1.3.3-1: Void

Table 7.1.3.5.6.1.3.3-1A: RRCReconfiguration (Step 1, Table 7.1.3.5.6.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig-DRB |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig-SCells |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.3.5.6.1.3.3-1B: RadioBearerConfig-DRB (Table 7.1.3.5.6.1.3.3-1A)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| srb3-ToRelease | Not present |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| drb-Identity | DRBi |  |  |
| pdcp-Config | PDCP-Config |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.5.6.1.3.3-1C: CellGroupConfig-SCells (Table 7.1.3.5.6.1.3.3-1A)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..max LC-ID)) OF RLC-BearerConfig { | 3 entries |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig-1 | entry 1 Table 7.1.3.5.6.1.3.3-3 RLC entity 1 (primary) |  |
| RLC-BearerConfig[2] | RLC-BearerConfig-2 | entry 2 Table 7.1.3.5.6.1.3.3-4 RLC entity 2 (secondary) |  |
| RLC-BearerConfig[3] | RLC-BearerConfig-3 | entry 3 Table 7.1.3.5.6.1.3.3-5 RLC entity 3 (secondary) |  |
| } |  |  |  |
| mac-CellGroupConfig | Not present |  |  |
| physicalCellGroupConfig | Not present |  |  |
| spCellConfig | Not present |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig { | 2 entries |  |  |
| SCellConfig[1] SEQUENCE { |  | entry 1 |  |
| sCellIndex | SCellIndex for NR Cell 3 |  |  |
| sCellConfigCommon | ServingCellConfigCommon as per TS 38.508-1 [4] Table 4.6.3-168 |  |  |
| sCellConfigDedicated | ServingCellConfig as per TS 38.508-1 [4] Table 4.6.3-167 |  |  |
| } |  |  |  |
| SCellConfig[2] SEQUENCE { |  | entry 2 |  |
| sCellIndex | SCellIndex for NR Cell 6 |  |  |
| sCellConfigCommon | ServingCellConfigCommon as per TS 38.508-1 [4] Table 4.6.3-168 |  |  |
| sCellConfigDedicated | ServingCellConfig as per TS 38.508-1 [4] Table 4.6.3-167 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.5.6.1.3.3-2: PDCP-Config (Table 7.1.3.5.6.1.3.3-1B)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-99 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-Config ::= SEQUENCE { |  |  |  |
| t-Reordering | Not present |  |  |
| moreThanTwoRLC-DRB-r16 SEQUENCE { |  |  |  |
| duplicationState-r16 SEQUENCE (SIZE (3)) | 3 bits |  |  |
| duplicationState-r16[1] | true |  |  |
| duplicationState-r16[2] | true |  |  |
| duplicationState-r16[3] | false |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.5.6.1.3.3-3: RLC-BearerConfig-1 (Table 7.1.3.5.6.1.3.3-1C)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-148 with Condition AM |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-BearerConfig ::= SEQUENCE { |  |  |  |
| logicalChannelIdentity | LogicalChannelIdentity with condition DRB j | ID of primary logical channel |  |
| servedRadioBearer CHOICE { |  |  |  |
| drb-Identity | DRB-Identity with condition DRB j | DRB |  |
| } |  |  |  |
| rlc-Config CHOICE { |  |  |  |
| am SEQUENCE { |  |  |  |
| ul-AM-RLC SEQUENCE { |  |  |  |
| maxRetxThreshold | t32 | To ensure RLC failure happens before RLF |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| mac-LogicalChannelConfig SEQUENCE { |  |  |  |
| ul-SpecificParameters SEQUENCE { |  |  |  |
| allowedServingCells SEQUENCE (SIZE (1..maxNrofServingCells-1)) OF ServCellIndex { | 1 entry |  |  |
| ServCellIndex[1] | ServCellIndex of NR Cell 1 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.5.6.1.3.3-4: RLC-BearerConfig-2 (Table 7.1.3.5.6.1.3.3-1C)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-148 with Condition AM |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-BearerConfig ::= SEQUENCE { |  |  |  |
| logicalChannelIdentity | LogicalChannelIdentity with condition DRB j+1 | To ensure ID of secondary logical channel ID is different with existing logical channel and the primary logical channel |  |
| servedRadioBearer CHOICE { |  |  |  |
| drb-Identity | DRB-Identity with condition DRB j | DRB |  |
| } |  |  |  |
| rlc-Config CHOICE { |  |  |  |
| am SEQUENCE { |  |  |  |
| ul-AM-RLC SEQUENCE { |  |  |  |
| maxRetxThreshold | t1 | To ensure RLC failure happens before RLF |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| mac-LogicalChannelConfig SEQUENCE { |  |  |  |
| ul-SpecificParameters SEQUENCE { |  |  |  |
| allowedServingCells SEQUENCE (SIZE (1..maxNrofServingCells-1)) OF ServCellIndex { | 1 entry |  |  |
| ServCellIndex[1] | ServCellIndex of NR Cell 3 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.5.6.1.3.3-5: RLC-BearerConfig-3 (Table 7.1.3.5.6.1.3.3-1C)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-148 with Condition AM |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-BearerConfig ::= SEQUENCE { |  |  |  |
| logicalChannelIdentity | LogicalChannelIdentity with condition DRB j+2 | To ensure ID of secondary logical channel ID is different with existing logical channel and the primary logical channel |  |
| servedRadioBearer CHOICE { |  |  |  |
| drb-Identity | DRB-Identity with condition DRB j | DRB |  |
| } |  |  |  |
| rlc-Config CHOICE { |  |  |  |
| am SEQUENCE { |  |  |  |
| ul-AM-RLC SEQUENCE { |  |  |  |
| maxRetxThreshold | t1 | To ensure RLC failure happens before RLF |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| mac-LogicalChannelConfig SEQUENCE { |  |  |  |
| ul-SpecificParameters SEQUENCE { |  |  |  |
| allowedServingCells SEQUENCE (SIZE (1..maxNrofServingCells-1)) OF ServCellIndex { | 1 entry |  |  |
| ServCellIndex[1] | ServCellIndex of NR Cell 6 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

###### 7.1.3.5.6.2	PDCP Duplication / 3 RLC entities / Intra-band non-Contiguous CA

The scope and description of the present TC is the same as test case 7.1.3.5.6.1 with the following differences:

-	CA configuration: Intra-band non-Contiguous CA replaces Intra-band Contiguous CA.

##### 7.1.3.5.7	Ethernet header compression and decompression / Correct functionality of ethernet header compression and decompression

7.1.3.5.7.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { SS sends PDCP Data PDU containing EHC compressed ethernet packet with Context ID A and with full ethernet header A to UE }

then { UE sends PDCP Control PDU to SS containing EHC feedback packet with Context ID A }

}

(2)

with { UE in RRC_CONNECTED state }

ensure that {

when { SS sends PDCP Data PDU containing EHC compressed ethernet packet with Context ID A and with full ethernet header A to UE }

then { UE sends PDCP Data PDU to SS containing EHC compressed ethernet packet with Context ID B and with full ethernet header A generated from the looped back ethernet packet }

}

(3)

with { UE in RRC_CONNECTED state }

ensure that {

when { SS sends PDCP Data PDU containing EHC compressed ethernet packet with Context ID A and without ethernet header to UE }

then { UE sends PDCP Data PDU to SS containing EHC compressed ethernet packet with Context ID B and with full ethernet header A generated from the looped back ethernet packet with decompressed ethernet header }

}

(4)

with { UE in RRC_CONNECTED state }

ensure that {

when { SS has sent PDCP Control PDU to UE containing EHC feedback packet with Context ID B and SS sends PDCP Data PDU to UE containing EHC compressed ethernet packet with Context ID A without ethernet header to UE }

then { UE sends PDCP Data PDU to SS containing EHC compressed ethernet packet with Context ID B and without ethernet header generated from the looped back ethernet packet with decompressed ethernet header }

}

7.1.3.5.7.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.323, clause 5.12. These are Rel-16 requirements.

[TS 38.323, clause 5.12.2]

PDCP entities associated with DRBs can be configured by upper layers TS 38.331 [3] to use EHC. Each PDCP entity carrying user plane data may be configured to use EHC. Every PDCP entity uses at most one EHC compressor instance and at most one EHC decompressor instance.

[TS 38.323, clause 5.12.3]

The usage and definition of the parameters shall be as specified below.

-	MAX_CID_EHC_UL: This is the maximum CID value that can be used for uplink. One CID value shall always be reserved for uncompressed flows. The parameter MAX_CID_EHC_UL is configured by upper layers (maxCID-EHC-UL in TS 38.331 [3]);

[TS 38.323, clause 5.12.4]

If EHC is configured, the EHC protocol generates two types of output packets:

-	EHC compressed packets (i.e. EHC full header packets and EHC compressed header packets), each associated with one PDCP SDU;

-	standalone packets not associated with a PDCP SDU, i.e. EHC feedback.

An EHC compressed packet is associated with the same PDCP SN and COUNT value as the related PDCP SDU. The header compression is not applicable to the SDAP header and the SDAP Control PDU if included in the PDCP SDU.

EHC feedback are not associated with a PDCP SDU. They are not associated with a PDCP SN and are not ciphered.

[TS 38.323, clause 5.12.5]

If EHC is configured by upper layers for PDCP entities associated with user plane data, the PDCP Data PDUs are decompressed by the EHC protocol after performing deciphering as explained in clause 5.8. The header decompression is not applicable to the SDAP header and the SDAP Control PDU if included in the PDCP Data PDU.

[TS 38.323, clause 5.12.6.1]

When an EHC feedback is generated by the EHC protocol, the transmitting PDCP entity shall:

-	submit to lower layers the corresponding PDCP Control PDU as specified in clause 6.2.3.3 i.e. without associating a PDCP SN, nor performing ciphering.

[TS 38.323, clause 5.12.6.2]

At reception of a PDCP Control PDU for EHC feedback from lower layers, the receiving PDCP entity shall:

-	deliver the corresponding EHC feedback to the EHC protocol without performing deciphering.

7.1.3.5.7.3	Test description

7.1.3.5.7.3.1	Pre-test conditions

Pre-test conditions as in clause 7.1.3.0 with condition pc_NG_RAN_NR, PDN/PDU session type Ethernet used in 4.5A.2 and message with condition UM used for step 7 in 4.5.4.2 according to [4] for primary DRB.

7.1.3.5.7.3.2	Test procedure sequence

Table 7.1.3.5.7.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits an RRCReconfiguration message to configure DRB to use ethernet header compression. | <-- | RRCReconfiguration | - | - |
| 2 | UE transmits RRCReconfigurationComplete message to confirm that reconfiguration with ethernet header compression configuration is completed. | --> | RRCReconfigurationComplete | - | - |
| 3 | SS transmits PDCP Data PDU containing EHC compressed ethernet packet with Context ID A and with full ethernet header information A. | <-- | PDCP Data PDU #0 | - | - |
| 4 | Check: Does UE transmit PDCP Control PDU containing unciphered EHC feedback packet with Context ID A? | --> | PDCP Control PDU #0 | 1 | P |
| 5 | Check: Does UE transmit PDCP Data PDU containing EHC compressed ethernet packet with Context ID B and with full ethernet header information A? | --> | PDCP Data PDU #0 | 2 | P |
| 6 | SS transmits PDCP Data PDU containing EHC compressed ethernet packet with Context ID A and without ethernet header information. | <-- | PDCP Control PDU #0 | - | - |
| 7 | Check: Does UE transmit PDCP Data PDU containing EHC compressed ethernet packet with Context ID B and with full ethernet header information A? | --> | PDCP Data PDU #1 | 3 | P |
| 8 | SS transmits PDCP Control PDU containing unciphered EHC feedback packet with Context ID B. | <-- | PDCP Control PDU #1 | - | - |
| 9 | SS transmits PDCP Data PDU containing EHC compressed ethernet packet with Context ID A and without ethernet header information. | <-- | PDCP Data PDU #2 | - | - |
| 10 | Check: Does UE transmit PDCP Data PDU containing EHC compressed ethernet packet with Context ID B and without ethernet header information? | --> | PDCP Data PDU #2 | 4 | P |
| Note 1: PDCP PDUs will be checked before and after decompressor on the NW side for test steps 5, 7 and 10. Note 2: The value of Context ID A and Context ID B can be any value. However, value of Context ID A and Context ID B shall stay the same in all steps. |  |  |  |  |  |

7.1.3.5.7.3.3	Specific message contents

Table 7.1.3.5.7.3.3-1: RRCReconfiguration (step 1, Table 7.1.3.5.7.3.2-1)

| Derivation Path: TS 38.508 [4], clause 4.6.1, table 4.6.1-13 RRCReconfiguration |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig-EHC |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.3.5.7.3.3-2: RadioBearerConfig-EHC (Table 7.1.3.5.7.3.3-1)

| Derivation Path: TS 38.331 [12], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| drb-Identity | DRB-Identity for DRB created in pre-test conditions |  |  |
| pdcp-Config | PDCP-Config-EHC |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.5.7.3.3-3: PDCP-Config-EHC

| Derivation Path: TS38.331 [12], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PDCP-Config ::= SEQUENCE { |  |  |  |  |
| ethernetHeaderCompression-r16 SEQUENCE { |  |  |  |  |
| ehc-Common-r16 SEQUENCE { |  |  |  |  |
| ehc-CID-Length-r16 |  | bits15 |  |  |
| } |  |  |  |  |
| ehc-Downlink-r16 SEQUENCE { |  |  |  |  |
| drb-ContinueEHC-DL-r16 |  | true |  |  |
| } |  |  |  |  |
| ehc-Uplink-r16 SEQUENCE { |  |  |  |  |
| maxCID-EHC-UL-r16 |  | 32767 |  |  |
| drb-ContinueEHC-UL-r16 |  | true |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

##### 7.1.3.5.8 to 7.1.3.5.11

##### 7.1.3.5.12	PDCP SN gap report / Receive operation

7.1.3.5.12.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and the associated PDCP t-Reordering timer is running }

ensure that {

when { UE receives PDCP SN gap Report, RX_DELIV is less than any COUNT value associated with the discarded PDCP SDUs but not equal to any COUNT value associated with the discarded PDCP SDUs }

then { UE does not update RX_DELIV, and UE does not deliver any stored PDCP SDU(s) with consecutively associated COUNT values starting from COUNT = RX_DELIV + 1 to upper layers }

}

(2)

with { UE in RRC_CONNECTED state and the associated PDCP t-Reordering timer is running }

ensure that {

when { t-Reordering expires }

then { UE updates RX_DELIV to the COUNT value of the first PDCP SDU with COUNT >= RX_REORD which has not been delivered to upper layers and is not considered as discarded, and UE deliver to the upper layers all stored PDCP SDU(s) with associated COUNT value(s) < RX_REORD and all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from RX_REORD, where consecutively associated COUNT value(s) include COUNT value(s) of both the stored PDCP SDU(s) and PDCP SDU(s) which are considered as discarded. }

}

(3)

with { UE in RRC_CONNECTED state and the associated PDCP t-Reordering timer is running }

ensure that {

when { UE receives PDCP SN gap Report, RX_DELIV is equal to any COUNT value associated with the discarded PDCP SDUs }

then { UE deliver all stored PDCP SDU(s) with consecutively associated COUNT values starting from COUNT = RX_DELIV + 1 to upper layers where consecutively associated COUNT value(s) include COUNT value(s) of both the stored PDCP SDU(s) and PDCP SDU(s) which are considered as discarded, and then updates RX_DELIV }

}

(4)

with { UE in RRC_CONNECTED state and the associated PDCP t-Reordering timer is not running }

ensure that {

when { UE receives PDCP SN gap Report, RX_DELIV is less than any COUNT value associated with the discarded PDCP SDUs but not equal to any COUNT value associated with the discarded PDCP SDUs, and RX_DELIV < RX_NEXT after updating RX_NEXT }

then { UE starts t-reordering and UE does not update RX_DELIV }

}

(5)

with { UE in RRC_CONNECTED state and the associated PDCP t-Reordering timer is running }

ensure that {

when { UE receives PDCP DATA COUNT = RX_DELIV }

then { UE deliver to the upper layers all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from RX_DELIV, where consecutively associated COUNT value(s) include COUNT value(s) of both the stored PDCP SDU(s) and PDCP SDU(s) which are considered as discarded, and then UE updates RX_DELIV to the COUNT value of the first PDCP SDU with COUNT value > RX_DELIV which has not been delivered to upper layers and is not considered as discarded.}

}

(6)

with { UE in RRC_CONNECTED state and the associated PDCP t-Reordering timer is not running }

ensure that {

when { UE receives PDCP SN gap Report, RX_DELIV is less than any COUNT value associated with the discarded PDCP SDUs but not equal to any COUNT value associated with the discarded PDCP SDUs, and RX_NEXT <= COUNT value associated with the last discarded PDCP SDU indicated in the PDCP SN gap report }

then { UE update RX_NEXT to the largest COUNT value associated with the discarded PDCP SDU + 1 }

}

7.1.3.5.12.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.323, clause 5.2.2.1, 5.2.2.2, 5.16.2, 6.1.2, 6.2.3.5, 6.3.8, 6.3.15 and 6.3.16. Unless otherwise stated these are Rel-18 requirements.

[TS 38.323, clause 5.2.2.1]

If the received PDCP Data PDU with COUNT value = RCVD_COUNT is not discarded above, the receiving PDCP entity shall:

…

-	if RCVD_COUNT = RX_DELIV:

-	deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before;

-	all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from COUNT = RX_DELIV, where consecutively associated COUNT value(s) include COUNT value(s) of both the stored PDCP SDU(s) and PDCP SDU(s) which are considered as discarded as specified in clause 5.16.2;

-	update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers and is not considered as discarded as specified in clause 5.16.2, with COUNT value > RX_DELIV;

[TS 38.323, clause 5.2.2.2]

When t-Reordering expires, the receiving PDCP entity shall:

-	deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before:

-	all stored PDCP SDU(s) with associated COUNT value(s) < RX_REORD, where consecutively associated COUNT value(s) include COUNT value(s) of both the stored PDCP SDU(s) and PDCP SDU(s) which are considered as discarded as specified in clause 5.16.2;

-	all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from RX_REORD;

-	update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers and is not considered as discarded as specified in clause 5.16.2, with COUNT value >= RX_REORD;

[TS 38.323, clause 5.16.2]

At reception of a PDCP SN gap report from lower layers, the receiving PDCP entity shall:

-	consider each PDCP SDU, if any, with the bit in the discard bitmap set to '1', or with the associated COUNT value equal to the value of FDC field as discarded;

-	if RX_DELIV is less than or equal to the largest COUNT value associated with the discarded PDCP SDUs:

-	if RX_NEXT <= COUNT value associated with the last discarded PDCP SDU indicated in the PDCP SN gap report:

-	update RX_NEXT to the largest COUNT value associated with the discarded PDCP SDU + 1;

-	if RX_DELIV is equal to any COUNT value associated with the discarded PDCP SDU(s):

-	deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before:

-	all stored PDCP SDU(s) with consecutively associated COUNT values starting from COUNT = RX_DELIV + 1, where consecutively associated COUNT value(s) include COUNT value(s) of both the stored PDCP SDU(s) and PDCP SDU(s) which are considered as discarded;

-	update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers and is not considered as discarded, with COUNT value > RX_DELIV;

-	if t-Reordering is running, and if RX_DELIV >= RX_REORD:

-	stop and reset t-Reordering;

-	if t-Reordering is not running (includes the case when t-Reordering is stopped due to actions above), and RX_DELIV < RX_NEXT:

-	update RX_REORD to RX_NEXT;

-	start t-Reordering.

[TS 38.323, clause 6.1.2]

The PDCP Control PDU is used to convey one of followings in addition to the PDU header:

…

-	a PDCP SN gap report.

[TS 38.323, clause 6.2.3.5]

Figure 6.2.3.5-1 shows the format of the PDCP control PDU carrying the PDCP SN gap report. This format is applicable for UM DRBs and AM DRBs.

Figure 6.2.3.5-1: PDCP Control PDU format for PDCP SN gap report

[TS 38.323, clause 6.3.8]

Length: 3 bits

This field indicates the type of control information included in the corresponding PDCP Control PDU.

Table 6.3.8-1: PDU type

| Bit | Description |
| --- | --- |
| 000 | PDCP status report |
| 001 | Interspersed ROHC feedback |
| 010 | EHC feedback |
| 011 | UDC feedback |
| 100 | PDCP SN gap report |
| 101-111 | Reserved |

[TS 38.323, clause 6.3.15]

Length: 32 bits

First discarded COUNT. This field indicates the smallest COUNT value among the COUNT value(s) associated with the discarded PDCP SDU(s).

[TS 38.323, clause 6.3.16]

Length: Variable. The length of the discard bitmap field can be 0.

This field indicates which SDUs are discarded and which SDUs are not discarded in the transmitting PDCP entity. The bit position of the Nth bit in the Discard Bitmap is N, i.e., the bit position of the first bit in the Discard Bitmap is 1.

Table 6.3.16-1 Discard Bitmap

| Bit | Description |
| --- | --- |
| 0 | PDCP SDU with COUNT = (FDC + bit position) modulo 232 is not discarded. |
| 1 | PDCP SDU with COUNT = (FDC + bit position) modulo 232 is discarded. |

7.1.3.5.12.3	Test description

7.1.3.5.12.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.3.0 exception of PDCP parameters according to Table 7.1.3.5.12.3.1-1.

Table 7.1.3.5.12.3.1-1: PDCP parameters

| t-Reordering | ms500 |
| --- | --- |

7.1.3.5.12.3.2	Test procedure sequence

Table 7.1.3.5.12.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS sends the PDCP SDU #0 D/C field = 1 (PDCP Data PDU) and PDCP SN = 0. | <-- | (PDCP SDU #0) | - | - |
| 2 | The UE transmits a PDCP SDU #0 with PDCP SN=0. | --> | (PDCP SDU #0) | - | - |
| 3 | The SS sends the PDCP SDU #2 D/C field = 1 (PDCP Data PDU) and PDCP SN = 2. The UE starts t-Reordering. Note T1. | <-- | (PDCP SDU #2) | - | - |
| 4 | Wait for 100ms (< configured t-Reordering) | - | - | - | - |
| 5 | The SS sends the PDCP SN gap Report indicating PDCP SDU #3 and SDU #4 are discarded. D/C field = 0 (PDCP Control PDU), PDU Type field = 100 (PDCP SN gap report), FDC field = 3 and Discard Bitmap1 field = 00000001’B. | <-- | (PDCP SN gap Report) | - | - |
| 6 | Check 1: Does the UE transmit the PDCP SDU #2 with PDCP SN=1 after t-Reordering expiry? Note T2 Check 2: Is (T2 – T1) > t-Reordering? | --> | (PDCP SDU #2) | 1,2 | P |
| 7 | The SS sends the PDCP SDU #5 D/C field = 1 (PDCP Data PDU) and PDCP SN = 5. | <-- | (PDCP SDU #5) | - | - |
| 8 | Check: Does the UE transmit a PDCP SDU #5 with PDCP SN=2? Note: UE updates RX_DELIV to 5 after step6. | --> | (PDCP SDU #5) | 2 | P |
| 9 | The SS sends the PDCP SDU #7 D/C field = 1 (PDCP Data PDU) and PDCP SN = 7. The UE starts t-Reordering. Note T3. | <-- | (PDCP SDU #7) | - | - |
| 10 | Wait for 100ms (< configured t-Reordering) | - | - | - | - |
| 11 | The SS sends the PDCP SN gap Report indicating PDCP SDU #3 and SDU #4 and SDU #6 and SDU #8 are discarded. D/C field = 0 (PDCP Control PDU), PDU Type field = 100’B (PDCP SN gap report), FDC field = 3 and Discard Bitmap1 field = 00010101’B. | <-- | (PDCP SN gap Report) | - | - |
| 12 | Check 1: Does the UE transmit the PDCP SDU #7 with PDCP SN=3? Note T4 Check 2: Is (T4 – T3) < t-Reordering? | --> | (PDCP SDU #7) | 3 | P |
| 13 | The SS sends the PDCP SDU #8 D/C field = 1 (PDCP Data PDU) and PDCP SN = 8. | <-- | (PDCP SDU #8) | - | - |
| 14 | Check: Does the UE transmit the PDCP SDU #8 in next 1s? Note: UE shall not transmit PDCP SDU #8 because RX_DELIV is updated to 9 after step 11. | --> | (PDCP SDU #8) | 3 | F |
| 15 | The SS sends the PDCP SN gap Report indicating PDCP SDU #3 and SDU #4 and SDU #6 and SDU #8 and SDU #10 are discarded. D/C field = 0 (PDCP Control PDU), PDU Type field = 100’B (PDCP SN gap report), FDC field = 3 and Discard Bitmap1 field = 01010101’B. The UE starts t-Reordering. Note T5. | <-- | (PDCP SN gap Report) | - | - |
| 16 | Wait for 100ms (< configured t-Reordering) | - | - | - | - |
| 17 | The SS sends the PDCP SDU #9 D/C field = 1 (PDCP Data PDU) and PDCP SN = 9. | <-- | (PDCP SDU #9) | - | - |
| 18 | Check 1: Does the UE transmit the PDCP SDU #9 with PDCP SN=4? Note T6 Check 2: Is (T6 – T5) < t-Reordering? | --> | (PDCP SDU #9) | 4,5 | P |
| 19 | The SS sends the PDCP SDU #10 D/C field = 1 (PDCP Data PDU) and PDCP SN = 10. | <-- | (PDCP SDU #10) | - | - |
| 20 | Check: Does the UE transmit the PDCP SDU #10 in next 1s? Note: UE shall not transmit PDCP SDU #10 because RX_DELIV is updated to 11 after step 17. | --> | (PDCP SDU #10) | 5 | F |
| 21 | The SS sends the PDCP SN gap Report indicating PDCP SDU #3 and SDU #4 and SDU #6 and SDU #8 and SDU #10 and SDU #12 are discarded. D/C field = 0 (PDCP Control PDU), PDU Type field = 100’B (PDCP SN gap report), FDC field = 3 and Discard Bitmap1 field = 0101010101’B. The UE starts t-Reordering. | <-- | (PDCP SN gap Report) | - | - |
| 22 | Wait for 600ms (>configured t-Reordering) | - | - | - | - |
| 23 | The SS sends the PDCP SDU #11 D/C field = 1 (PDCP Data PDU) and PDCP SN = 11. | <-- | (PDCP SDU #11) | - | - |
| 24 | Check: Does the UE transmit the PDCP SDU #11 in next 1s? Note: UE shall not transmit PDCP SDU #11, because RX_NEXT is updated to 13 after step 21, and then t-Reordering started due to RX_NEXT > RX_DELIV. When t-Reordering expire, RX_DELIV is updated to 13. | --> | (PDCP SDU #11) | 6 | F |

7.1.3.5.12.3.3	Specific message contents

None.

##### 7.1.3.5.13	LTM cell switch / Master Key / PDCP persists

7.1.3.5.13.1	Test Purpose (TP)

(1)

with { UE in RRC_Connected state with candidate configuration for LTM }

ensure that {

when { UE receives LTM Cell Switch Command MAC CE }

then { UE triggers RACH procedure on LTM Cell }

}

(2)

with { UE in RRC_Connected state with candidate configuration for LTM }

ensure that {

when { UE receives RRCReconfiguration with new candidate configuration for LTM }

then { UE takes the new configuration into account at the next LTM cell switch }

}

7.1.3.5.13.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331 clauses 5.3.5.18.6 and 5.3.1.2, TS 38.323 clauses 5.2.2.1, 5.2.2.2, 5.2.2.3, 7.1 and 7.3. Unless otherwise stated these are Rel-18 requirements.

[TS 38.331, clause 5.3.5.18.6]

Upon the indication by lower layers that an LTM cell switch procedure is triggered, or upon performing LTM cell switch following cell selection performed while timer T311 was running, as specified in 5.3.7.3, the UE shall:

1>	if the LTM cell switch is triggered on the MCG:

2>	release/clear all current dedicated and common radio configurations which have neither been received via SRB1 within mrdc-SecondaryCellGroup, nor via SRB3 except for the following:

-	the radio bearer configuration (configured via RadioBearerConfig)

-	the logicalChannelIdentity and logicalChannelIdentityExt of RLC bearers configured in RLC-BearerConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the bh-LogicalChannelIdentity of BH RLC channels configured in BH-RLC-ChannelConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the UE variables VarLTM-ServingCellNoResetID and VarLTM-ServingCellUE-MeasuredTA-ID;

-	the ltm-Config;

-	the MCG C-RNTI;

-	the AS security configurations associated with the master key;

-	the logged measurement configuration;

1>	else, if the LTM cell switch is triggered on the SCG:

2>	release/clear all current dedicated and common radio configurations which have been received either via SRB1 within mrdc-SecondaryCellGroup, or via SRB3 except for the following:

-	the radio bearer configuration (configured via RadioBearerConfig IE)

-	the logicalChannelIdentity and logicalChannelIdentityExt of RLC bearers configured in RLC-BearerConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the bh-LogicalChannelIdentity of BH RLC channels configured in BH-RLC-ChannelConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the UE variables VarLTM-ServingCellNoResetID and VarLTM-ServingCellUE-MeasuredTA-ID;

-	the ltm-Config;

-	the AS security configurations associated with the secondary key;

1>	for each SRB/DRB in the current UE configuration:

2>	if the LTM cell switch is triggered on the MCG and the SRB/DRB using the master key; or

2>	if the LTM cell switch is triggered on the SCG and the SRB/DRB using the secondary key:

3>	keep the associated PDCP and SDAP entities, their state variables, buffers and timers;

3>	release all fields related to the SRB/DRB configuration except for srb-Identity and drb-Identity;

3>	apply the default SRB configuration defined in 9.2.1 for the corresponding SRB;

NOTE 00:	For all radio bearers and RLC bearers included in the LTM candidate configuration to be applied at an LTM cell switch execution (i.e., as derived from the LTM reference configuration and the LTM candidate configuration), even if those radio bearers and RLC bearers were configured before the LTM cell switch execution, the network includes fields as specified for the initial setup of radio bearers and RLC bearers and sets the values previously stored by the UE for the fields that cannot be modified according to presence conditions or field descriptions.

1>	apply the default L1 parameter values as specified in corresponding physical layer specifications except for the parameters for which values are provided in SIB1;

1>	use the default values specified in 9.2.3 for timers T310, T311 and constants N310, N311 associated with the cell group for which the LTM cell switch procedure is triggered, where T310, N310, and N311 are for both MCG and SCG, and T311 is only for the MCG;

1>	apply the default MAC Cell Group configuration as specified in 9.2.2 for the cell group for which the LTM cell switch procedure is triggered;

1>	if the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 does not contain the field ltm-NoResetID and if the UE does not have any value stored of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID; or

1>	if the value of field ltm-NoResetID contained within the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID:

2>	for each logicalChannelIdentity and logicalChannelIdentityExt that is part of the current UE configuration for the cell group for which the LTM cell switch procedure is triggered:

3>	if servedRadioBearer is set to drb-Identity:

4>	after the end of this procedure, re-establish the corresponding RLC entity as specified in TS 38.322 [4], after applying the LTM configuration in ltm-CandidateConfig within the LTM-Candidate IE in ltm-Config;

2>	for each bh-LogicalChannelIdentity that is part of the current UE configuration for the cell group for which the LTM cell switch procedure is triggered:

3>	after the end of this procedure, re-establish the corresponding RLC entity as specified in TS 38.322 [4], after applying the LTM configuration in ltm-CandidateConfig within the LTM-Candidate IE in ltm-Config;

2>	for each drb-Identity value that is part of the current UE configuration:

3>	if this DRB is an AM DRB:

4>	after the end of this procedure, trigger the PDCP entity of this DRB to perform data recovery as specified in TS 38.323 [5], after applying the LTM configuration in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config;

2>	if the value of field ltm-NoResetID contained within the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID:

3>	replace the value of ltm-ServingCellNoResetID in VarLTM-ServingCellNoResetID with the value of ltm-NoResetID in the LTM-Candidate in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3;

1> if the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 contains the field ltm-UE-MeasuredTA-ID:

2>	if the value of ltm-UE-MeasuredTA-ID is not equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID:

3>	replace the value of ltm-ServingCellUE-MeasuredTA-ID in VarLTM-ServingCellUE-MeasuredTA-ID with the value received within ltm-UE-MeasuredTA-ID;

3>	for each LTM-Candidate IE in ltm-Config:

4>	if the value of ltm-UE-MeasuredTA-ID within LTM-Candidate IE is equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID:

5>	inform lower layers that the UE is configured with UE-based TA measurements for the LTM-Candidate;

4>	else:

5>	inform lower layers that the UE is not configured with UE-based TA measurements for the LTM-Candidate;

NOTE 0:	The UE is not expected to perform UE-based TA measurements for an SpCell.

1>	if ltm-ConfigComplete is not included within the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3:

2>	consider ltm-ReferenceConfiguration in ltm-Config, associated with the cell group for which the LTM cell switch procedure is triggered, to be the current UE configuration for the fields and configurations to be released by the actions above in this procedure;

2>	if measConfig is included within ltm-ReferenceConfiguration in ltm-Config;

3>	perform the measurement configuration procedure as specified in clause 5.5.2 by considering the measConfig within ltm-ReferenceConfiguration in ltm-Config as the received measConfig:

NOTE 1:	When the UE considers the reference configuration to be the current UE configuration, the UE should store fields and configurations that are part of the reference configuration but should not execute any actions or procedures triggered by the reception of an RRCReconfiguration message which are described in clause 5.3.5.3, unless specified otherwise in this clause.

1>	if the LTM cell switch is triggered by an indication from lower layers:

2>	apply the RRCReconfiguration message in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config identified by the LTM candidate configuration identity received from lower layers according to clause 5.3.5.3;

1>	else (LTM cell switch triggered upon cell selection performed while timer T311 was running):

2>	apply the RRCReconfiguration message in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config related to the LTM candidate configuration identity for the selected cell (i.e., in accordance with 5.3.7.3) according to clause 5.3.5.3;

1>	if the LTM cell switch is triggered on the MCG:

2>	release the radio bearer(s) using the master key and the MCG logical channel(s) that were part of the UE configuration before this LTM cell switch procedure but not part of the LTM candidate configuration either indicated by lower layers or for the selected cell in accordance with 5.3.7.3, or the LTM reference configuration (in case the LTM candidate configuration does not include ltm-ConfigComplete);

1>	else, if the LTM cell switch is triggered on the SCG:

2>	release the radio bearer(s) using the secondary key and the SCG logical channel(s) that were part of the UE configuration before this LTM cell switch procedure but not part of the LTM candidate configuration either indicated by lower layers or for the selected cell in accordance with 5.3.7.3, or the LTM reference configuration (in case the LTM candidate configuration does not include ltm-ConfigComplete);

NOTE 2:	When ltm-ConfigComplete is not included for an LTM candidate configuration, before an LTM cell switch is triggered a UE implementation may generate and store an RRCReconfiguration message by applying the received LTM candidate configuration on top of the LTM reference configuration, and the stored RRCReconfiguration message is applied when the LTM cell switch is triggered. It is up to the UE to ensure that the RRC reconfiguration applied at the time of LTM cell switch is in accordance with the latest LTM reference configuration and LTM candidate configuration.

[TS 38.323, clause 5.2.2.1]

In this clause, following definitions are used:

-	HFN(State Variable): the HFN part (i.e. the number of most significant bits equal to HFN length) of the State Variable;

-	SN(State Variable): the SN part (i.e. the number of least significant bits equal to PDCP SN length) of the State Variable;

-	RCVD_SN: the PDCP SN of the received PDCP Data PDU, included in the PDU header;

-	RCVD_HFN: the HFN of the received PDCP Data PDU, calculated by the receiving PDCP entity;

-	RCVD_COUNT: the COUNT of the received PDCP Data PDU = [RCVD_HFN, RCVD_SN].

At reception of a PDCP Data PDU from lower layers, the receiving PDCP entity shall determine the COUNT value of the received PDCP Data PDU, i.e. RCVD_COUNT, as follows:

-	if RCVD_SN < SN(RX_DELIV) – Window_Size:

-	RCVD_HFN = HFN(RX_DELIV) + 1.

-	else if RCVD_SN >= SN(RX_DELIV) + Window_Size:

-	RCVD_HFN = HFN(RX_DELIV) – 1.

-	else:

-	RCVD_HFN = HFN(RX_DELIV);

-	RCVD_COUNT = [RCVD_HFN, RCVD_SN].

After determining the COUNT value of the received PDCP Data PDU = RCVD_COUNT, the receiving PDCP entity shall:

-	perform deciphering and integrity verification of the PDCP Data PDU using COUNT = RCVD_COUNT;

-	if integrity verification fails:

-	indicate the integrity verification failure to upper layer;

-	discard the PDCP Data PDU and consider it as not received;

-	if RCVD_COUNT < RX_DELIV; or

-	if the PDCP Data PDU with COUNT = RCVD_COUNT has been received before:

-	discard the PDCP Data PDU;

If the received PDCP Data PDU with COUNT value = RCVD_COUNT is not discarded above, the receiving PDCP entity shall:

-	store the resulting PDCP SDU in the reception buffer;

-	if RCVD_COUNT >= RX_NEXT:

-	update RX_NEXT to RCVD_COUNT + 1.

-	if outOfOrderDelivery is configured:

-	deliver the resulting PDCP SDU to upper layers after performing header decompression using EHC.

-	if RCVD_COUNT = RX_DELIV:

-	deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before;

-	all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from COUNT = RX_DELIV, where consecutively associated COUNT value(s) include COUNT value(s) of both the stored PDCP SDU(s) and PDCP SDU(s) which are considered as discarded as specified in clause 5.16.2;

-	update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers and is not considered as discarded as specified in clause 5.16.2, with COUNT value > RX_DELIV;

-	if t-Reordering is running, and if RX_DELIV >= RX_REORD:

-	stop and reset t-Reordering.

-	if t-Reordering is not running (includes the case when t-Reordering is stopped due to actions above), and RX_DELIV < RX_NEXT:

-	update RX_REORD to RX_NEXT;

-	start t-Reordering.

[TS 38.323, clause 5.2.2.2]

When t-Reordering expires, the receiving PDCP entity shall:

-	deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before:

-	all stored PDCP SDU(s) with associated COUNT value(s) < RX_REORD;

-	all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from RX_REORD, where consecutively associated COUNT value(s) include COUNT value(s) of both the stored PDCP SDU(s) and PDCP SDU(s) which are considered as discarded as specified in clause 5.16.2;

-	update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers and is not considered as discarded as specified in clause 5.16.2, with COUNT value >= RX_REORD;

-	if RX_DELIV < RX_NEXT:

-	update RX_REORD to RX_NEXT;

-	start t-Reordering.

[TS 38.323, clause 5.2.2.3]

When the value of the t-Reordering is reconfigured by upper layers while the t-Reordering is running, the receiving PDCP entity shall:

-	update RX_REORD to RX_NEXT;

-	stop and restart t-Reordering.

[TS 38.331, clause 5.3.1.2]

For a UE provided with an sk-counter, keyToUse indicates whether the UE uses the master key (KgNB) or the secondary key (S-KeNB or S-KgNB) for a particular DRB. The secondary key is derived from the master key and sk-Counter, as defined in TS 33.501[11]. Whenever there is a need to refresh the secondary key, e.g. upon change of MN with KgNB change or to avoid COUNT reuse, the security key update is used (see 5.3.5.7). When the UE is in NR-DC, the network may provide a UE configured with an SCG with an sk-Counter even when no DRB is setup using the secondary key (S-KgNB) in order to allow the configuration of SRB3. The network can also provide the UE with an sk-Counter, even if no SCG is configured, when using SN terminated MCG bearers.

[TS 38.323, clause 7.1]

This clause describes the state variables used in PDCP entities in order to specify the PDCP protocol. The state variables defined in this clause are normative.

All state variables are non-negative integers, and take values from 0 to [232 – 1].

PDCP Data PDUs are numbered integer sequence numbers (SN) cycling through the field: 0 to [2[pdcp-SN-SizeUL] – 1] or 0 to [2[pdcp-SN-SizeDL] – 1] or 0 to [2[sl-PDCP-SN-Size] – 1].

The transmitting PDCP entity shall maintain the following state variables:

a)	TX_NEXT

This state variable indicates the COUNT value of the next PDCP SDU to be transmitted. The initial value is 0, except for SRBs configured with state variables continuation. For target SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding source SRB. For source SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding target SRB.

The receiving PDCP entity shall maintain the following state variables:

a)	RX_NEXT

This state variable indicates the COUNT value of the next PDCP SDU expected to be received. The initial value is 0, except for sidelink broadcast and groupcast, for SRBs configured with state variables continuation, for multicast MRBs whose PDCP COUNT is not synchronized as indicated by upper layer, and for broadcast MRBs. For NR sidelink communication for broadcast and groupcast or sidelink SRB4 for NR sidelink discovery, the initial value of the SN part of RX_NEXT is (x +1) modulo (2[sl-PDCP-SN-Size]), where x is the SN of the first received PDCP Data PDU. For multicast MRBs whose PDCP COUNT is not synchronized as indicated by upper layer, and for broadcast MRBs, the initial value of the SN part of RX_NEXT is (x +1) modulo (2[PDCP-SN-SizeDL]), where x is the SN of the first received PDCP Data PDU. For target SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding source SRB. For source SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding target SRB.

NOTE 1:	For NR sidelink communication for broadcast and groupcast or sidelink SRB4 for NR sidelink discovery, it is up to UE implementation to select the HFN part for RX_NEXT such that initial value of RX_DELIV should be a positive value.

NOTE 2:	For multicast MRBs whose PDCP COUNT is not synchronized as indicated by upper layer, and for broadcast MRBs, the initial value of the HFN part of RX_NEXT is set by UE implementation.

b)	RX_DELIV

This state variable indicates the COUNT value of the first PDCP SDU not delivered to the upper layers, but still waited for. The initial value is 0, except for sidelink broadcast and groupcast, for SRBs configured with state variables continuation, and for MRBs. For NR sidelink communication for broadcast and groupcast or sidelink SRB4 for NR sidelink discovery, the initial value of the SN part of RX_DELIV is (x – 0.5 × 2[sl-PDCP-SN-Size–1]) modulo (2[sl-PDCP-SN-Size]), where x is the SN of the first received PDCP Data PDU. For multicast MRBs whose PDCP COUNT is not synchronized as indicated by upper layer, and for broadcast MRBs, the initial value of the SN part of RX_DELIV is set to (x – 0.5 × 2[PDCP-SN-SizeDL–1]) modulo (2[PDCP-SN-SizeDL]), where x is the SN of the first received PDCP Data PDU. For multicast MRBs, the initial value of RX_DELIV is set, if provided, by initialRX-DELIV in TS 38.331 [3]. For target SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding source SRB. For source SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding target SRB.

NOTE 3:	For multicast MRBs whose PDCP COUNT is not synchronized as indicated by upper layer, and for broadcast MRBs, the initial value of the HFN part of RX_DELIV is set by UE implementation.

c)	RX_REORD

This state variable indicates the COUNT value following the COUNT value associated with the PDCP Data PDU which triggered t-Reordering. For target SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding source SRB. For source SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding target SRB.

[TS 38.323, clause 7.3]

The transmitting PDCP entity shall maintain the following timers:

a) discardTimer

This timer is configured only for DRBs. The duration of the timer is configured by upper layers TS 38.331 [3]. In the transmitter, a new timer is started upon reception of an SDU from upper layer as specified in clause 5.2.1.

b) discardTimerForLowImportance

This timer is configured only for DRBs. The duration of the timer is configured by upper layers TS 38.331 [3]. In the transmitter, a new timer is started upon reception of an SDU belonging to a low importance PDU Set from upper layer as specified in clause 5.2.1.

The receiving PDCP entity shall maintain the following timers:

c) t-Reordering

The duration of the timer is configured by upper layers TS 38.331 [3], except for the case of NR sidelink communication or sidelink SRB4. For NR sidelink communication or sidelink SRB4, the t-Reordering timer is determined by the UE implementation. This timer is used to detect loss of PDCP Data PDUs as specified in clause 5.2.2. If t-Reordering is running, t-Reordering shall not be started additionally, i.e. only one t-Reordering per receiving PDCP entity is running at a given time.

7.1.3.5.13.3	Test description

7.1.3.5.13.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.3.0 with the exception except the following:

-	3 NR cells (NR Cell 1, NR Cell 2 and NR Cell 10) are configured. NR Cell 1 is the PCell, NR Cell 10 is the PSCell, neighbour cell NR Cell 2 is on the same frequency as the PCell.

-	The UE is in state RRC_CONNECTED using generic procedure parameter Connectivity (NR-DC), Bearers (MCG(s) and SCG) established according to TS 38.508-1 [4], clause 4.5.4. The UE uses the master key for MCG DRB.

-	AM DRB is configured according to Table 7.1.3.5.13.3.1-1.

Table 7.1.3.5.13.3.1-1: PDCP parameters

| Parameter | Value |
| --- | --- |
| t-Reordering | ms750 |
| statusReportRequired | Not present |

7.1.3.5.13.3.2	Test procedure sequence

Tables 7.1.3.5.13.3.2-1 and 7.1.3.5.13.3.2-2 illustrate the downlink power levels to be applied for NR Cell 1, NR Cell 2 and NR Cell 10 at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while the configuration marked "T1" is applied at the point indicated in the Main behaviour description in Table 7.1.3.5.13.3.2-3.

Table 7.1.3.5.13.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 10 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -88 | -88 | -94 | Power levels are such that entry condition for event A3 is not satisfied for the neighbour NR cell |
| T1 | SS/PBCH SSS EPRE | dBm/ SCS | -88 | -88 | -82 | Power levels are such that entry condition for event A3 is satisfied for intra-frequency neighbour NR cell |

Table 7.1.3.5.13.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 10 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -91 | -91 | -100 | Power levels are such that entry condition for event A3 is not satisfied for the neighbour NR cell |
| T1 | SS/PBCH SSS EPRE | dBm/ SCS | -91 | -91 | -82 | Power levels are such that entry condition for event A3 is satisfied for intra-frequency neighbour NR cell |

Table 7.1.3.5.13.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message on NR Cell 1 including MeasConfig to setup NR measurement and reporting for intra-frequency event A3. | <-- | RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message containing RRCReconfigurationComplete message on NR Cell 1. | --> | RRCReconfigurationComplete | - | - |
| 3 | The SS sends the PDCP SDU #1 D/C field = 1 (PDCP Data PDU) and PDCP SN = 1 on NR Cell 1. | <-- | (PDCP SDU #1) | - | - |
| 4 | The SS sends the PDCP SDU #2 D/C field = 1 (PDCP Data PDU) and PDCP SN =2 on NR Cell 1. | <-- | (PDCP SDU #2) | - | - |
| 5 | SS re-adjusts the cell-specific reference signal level according to row "T1" in table 7.1.3.5.13.3.2-1/2. | - | - | - | - |
| 6 | The UE transmits a MeasurementReport message embedded in NR RRC message ULInformationTransferMRDC to report event A3 with the measured RSRP for NR Cell 2 on NR Cell 1. | --> | MeasurementReport | - | - |
| 7 | The SS transmits an RRCReconfiguration message on NR Cell 1 containing LTM-Candidate-r18 for NR Cell 2. | <-- | RRCReconfiguration | - | - |
| 8 | The UE transmits an RRCReconfigurationComplete message on NR Cell 1. | --> | RRCReconfigurationComplete | - | - |
| 9 | The SS transmits an RRCReconfiguration message on NR Cell 1 with t-Reordering set to ms160 for NR Cell 2. | <-- | RRCReconfiguration | - | - |
| 10 | The UE transmits an RRCReconfigurationComplete message on NR Cell 1. | --> | RRCReconfigurationComplete | - | - |
| 11 | The SS transmits the candidate cell TCI activation MAC CE with Candidate Cell ID set to 0 for NR Cell 2 on NR Cell 1. | <-- | MAC PDU (Candidate Cell TCI state activation/deactivation MAC CE) | - | - |
| 12 | The SS transmits DCI 1_0 PDCCH order to trigger the UE PRACH transmission on NR Cell 1. | <-- | (PDCCH Order) | - | - |
| 13 | Check: Does UE transmits the PRACH preamble on NR Cell 2? | --> | (PRACH Preamble) | 1 | P |
| 14 | The SS transmits LTM cell switch command MAC CE with Timing Advance Command value set to 0 and Target Configuration ID value set to 0 to the UE on NR Cell 1. | <-- | MAC PDU (LTM Cell Switch Command MAC CE) | - | - |
| 15 | The UE transmits an RRCReconfigurationComplete message on NR Cell 2. | --> | RRCReconfigurationComplete | - | - |
| 16 | Wait 160ms from step 3 until t-Reordering expires. | - | - | - | - |
| 17 | The SS sends the PDCP SDU #0 D/C field = 1 (PDCP Data PDU) and PDCP SN = 0 on NR Cell 2. | <-- | (PDCP SDU #0) | - | - |
| - | EXCEPTION: Steps 18-20 below can occur in any order in the next 1 second. | - | - | - | - |
| 18 | Check: Does the UE transmit the PDCP SDU #0 via the AM RLC entity D/C field = 1 (PDCP Data PDU) and PDCP SN = 0 on NR Cell 2? | --> | (PDCP SDU #0) | 2 | F |
| 19 | Check: Does the UE transmit the PDCP SDU #1 via the AM RLC entity D/C field = 1 (PDCP Data PDU) and PDCP SN = 1 on NR Cell 2? | --> | (PDCP SDU #1) | 2 | F |
| 20 | Check: Does the UE transmit the PDCP SDU #2 via the AM RLC entity D/C field = 1 (PDCP Data PDU) and PDCP SN = 2 on NR Cell 2? | --> | (PDCP SDU #2) | 2 | F |

7.1.3.5.13.3.3	Specific message contents

Table 7.1.3.5.13.3.3-1: RRCReconfiguration (step 1, Table 7.1.3.5.13.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with conditions NR_MEAS

Table 7.1.3.5.13.3.3-2: MeasConfig (Table 7.1.3.5.13.3.3-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR | Table 7.1.3.5.13.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-EventA3 | Table 7.1.3.5.13.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.5.13.3.3-3: MeasObjectNR (Table 7.1.3.5.13.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR ::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for SSB of NR Cell 1 |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| nrofSS-BlocksToAverage | Not present |  |  |
| } |  |  |  |

Table 7.1.3.5.13.3.3-4: ReportConfigNR-EventA3 (Table 7.1.3.5.13.3.3-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-142 with condition EVENT_A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | 2 | 1 dB(2*0.5 dB) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportAmount | infinity |  |  |
| maxNrofRS-IndexesToReport | 1 |  |  |
| includeBeamMeasurements | true |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | true |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.5.13.3.3-5: MeasurementReport (step 7, Table 7.1.3.5.13.3.2-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-5A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| measurementReport SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServingMOList SEQUENCE (SIZE (1..maxNrofServingCells)) OF MeasResultServMO { | 1 entry |  |  |
| MeasResultServMO[1] SEQUENCE { |  | entry 1 |  |
| servCellId | ServCellIndex of NR Cell 1 |  |  |
| measResultServingCell SEQUENCE { |  |  |  |
| physCellId | PCI of NR Cell 1 |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not present |  |  |
|  | Not checked |  | pc_ss_SINR_Meas |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListNR SEQUENCE (SIZE (1.. maxCellReport)) OF MeasResultNR { | 1 entry |  |  |
| MeasResultNR[1] SEQUENCE { |  | entry 1 |  |
| physCellId | Physical layer cell identity of NR Cell 2 |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | Not present |  |  |
| sinr | Not present |  |  |
| } |  |  |  |
| resultsCSI-RS-Cell | Not present |  |  |
| } |  |  |  |
| rsIndexResults | Not present |  |  |
| } |  |  |  |
| cgi-Info | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.5.13.3.3-6: RRCReconfiguration (step 8, Table 7.1.3.5.13.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with conditions LTM

Table 7.1.3.5.13.3.3-7: LTM-Config-r18(Table 7.1.3.5.13.3.3-6)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-67D |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-Config-r18 ::= SEQUENCE { |  |  |  |
| ltm-CandidateToAddModList-r18 SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-Candidate-r18 { | 1 entry |  |  |
| LTM-Candidate-r18[1] | LTM-Candidate-r18 | Table 7.1.3.5.13.3.3-8 |  |
| } |  |  |  |
| ltm-CSI-ResourceConfig-r18 | Not present |  |  |
| } |  |  |  |

Table 7.1.3.5.13.3.3-8: LTM-Candidate-r18 (Table 7.1.3.5.13.3.3-7)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-67C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-Candidate-r18 ::= SEQUENCE { |  |  |  |
| ltm-CandidateId-r18 | 1 |  |  |
| ltm-CandidatePCI-r18 | Physical layer cell identity of NR Cell 2 |  |  |
| ltm-CandidateConfig-r18 OCTET STRING (CONTAINING RRCReconfiguration) { |  |  |  |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| secondaryCellGroup | CellGroupConfig specified in TS 38.508-1 [4] Table 4.6.3-19 with condition NR-DC_SCG |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ltm-TCI-Info-r18 | LTM-TCI-Info specified in TS 38.508-1 [4] Table 4.6.3-67I with condition Joint |  |  |
| } |  |  |  |

##### 7.1.3.5.14	LTM cell switch / MCG / Master Key / PDCP persists / NR-DC

7.1.3.5.14.1	Test Purpose (TP)

(1)

with { UE in RRC_Connected state with candidate configuration for LTM MCG and the SRB/DRB are using the master key }

ensure that {

when { UE receives Candidate Cell TCI States Activation followed by LTM Cell Switch Command MAC CE for a MAC entity associated with MCG using same MasterKey}

then { UE skips the Random Access procedure for the LTM cell switch, performs LTM Cell Switch and mantains PDCP Discard Timer}

}

7.1.3.5.1.4.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331 clause 5.3.5.18.6, TS 38.321 clause 5.18.35 and TS 38.323, clause 5.3, 7.1, 7.2 and 7.3. Unless otherwise stated these are Rel-18 requirements.

[TS 38.331, clause 5.3.5.18.6]

Upon the indication by lower layers that an LTM cell switch procedure is triggered, or upon performing LTM cell switch following cell selection performed while timer T311 was running, as specified in 5.3.7.3, the UE shall:

1>	if the LTM cell switch is triggered on the MCG:

2>	release/clear all current dedicated and common radio configurations which have neither been received via SRB1 within mrdc-SecondaryCellGroup, nor via SRB3 except for the following:

-	the radio bearer configuration (configured via RadioBearerConfig)

-	the logicalChannelIdentity and logicalChannelIdentityExt of RLC bearers configured in RLC-BearerConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the bh-LogicalChannelIdentity of BH RLC channels configured in BH-RLC-ChannelConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the UE variables VarLTM-ServingCellNoResetID and VarLTM-ServingCellUE-MeasuredTA-ID;

-	the ltm-Config;

-	the MCG C-RNTI;

-	the AS security configurations associated with the master key;

-	the logged measurement configuration;

1>	else, if the LTM cell switch is triggered on the SCG:

2>	release/clear all current dedicated and common radio configurations which have been received either via SRB1 within mrdc-SecondaryCellGroup, or via SRB3 except for the following:

-	the radio bearer configuration (configured via RadioBearerConfig IE)

-	the logicalChannelIdentity and logicalChannelIdentityExt of RLC bearers configured in RLC-BearerConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the bh-LogicalChannelIdentity of BH RLC channels configured in BH-RLC-ChannelConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the UE variables VarLTM-ServingCellNoResetID and VarLTM-ServingCellUE-MeasuredTA-ID;

-	the ltm-Config;

-	the AS security configurations associated with the secondary key;

1>	for each SRB/DRB in the current UE configuration:

2>	if the LTM cell switch is triggered on the MCG and the SRB/DRB using the master key; or

2>	if the LTM cell switch is triggered on the SCG and the SRB/DRB using the secondary key:

3>	keep the associated PDCP and SDAP entities, their state variables, buffers and timers;

3>	release all fields related to the SRB/DRB configuration except for srb-Identity and drb-Identity;

3>	apply the default SRB configuration defined in 9.2.1 for the corresponding SRB;

NOTE 00:	For all radio bearers and RLC bearers included in the LTM candidate configuration to be applied at an LTM cell switch execution (i.e., as derived from the LTM reference configuration and the LTM candidate configuration), even if those radio bearers and RLC bearers were configured before the LTM cell switch execution, the network includes fields as specified for the initial setup of radio bearers and RLC bearers and sets the values previously stored by the UE for the fields that cannot be modified according to presence conditions or field descriptions.

1>	apply the default L1 parameter values as specified in corresponding physical layer specifications except for the parameters for which values are provided in SIB1;

1>	use the default values specified in 9.2.3 for timers T310, T311 and constants N310, N311 associated with the cell group for which the LTM cell switch procedure is triggered, where T310, N310, and N311 are for both MCG and SCG, and T311 is only for the MCG;

1>	apply the default MAC Cell Group configuration as specified in 9.2.2 for the cell group for which the LTM cell switch procedure is triggered;

1>	if the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 does not contain the field ltm-NoResetID and if the UE does not have any value stored of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID; or

1>	if the value of field ltm-NoResetID contained within the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID:

2>	for each logicalChannelIdentity and logicalChannelIdentityExt that is part of the current UE configuration for the cell group for which the LTM cell switch procedure is triggered:

3>	if servedRadioBearer is set to drb-Identity:

4>	after the end of this procedure, re-establish the corresponding RLC entity as specified in TS 38.322 [4], after applying the LTM configuration in ltm-CandidateConfig within the LTM-Candidate IE in ltm-Config;

2>	for each bh-LogicalChannelIdentity that is part of the current UE configuration for the cell group for which the LTM cell switch procedure is triggered:

3>	after the end of this procedure, re-establish the corresponding RLC entity as specified in TS 38.322 [4], after applying the LTM configuration in ltm-CandidateConfig within the LTM-Candidate IE in ltm-Config;

2>	for each drb-Identity value that is part of the current UE configuration:

3>	if this DRB is an AM DRB:

4>	after the end of this procedure, trigger the PDCP entity of this DRB to perform data recovery as specified in TS 38.323 [5], after applying the LTM configuration in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config;

2>	if the value of field ltm-NoResetID contained within the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID:

3>	replace the value of ltm-ServingCellNoResetID in VarLTM-ServingCellNoResetID with the value of ltm-NoResetID in the LTM-Candidate in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3;

1> if the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 contains the field ltm-UE-MeasuredTA-ID:

2>	if the value of ltm-UE-MeasuredTA-ID is not equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID:

3>	replace the value of ltm-ServingCellUE-MeasuredTA-ID in VarLTM-ServingCellUE-MeasuredTA-ID with the value received within ltm-UE-MeasuredTA-ID;

3>	for each LTM-Candidate IE in ltm-Config:

4>	if the value of ltm-UE-MeasuredTA-ID within LTM-Candidate IE is equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID:

5>	inform lower layers that the UE is configured with UE-based TA measurements for the LTM-Candidate;

4>	else:

5>	inform lower layers that the UE is not configured with UE-based TA measurements for the LTM-Candidate;

NOTE 0:	The UE is not expected to perform UE-based TA measurements for an SpCell.

1>	if ltm-ConfigComplete is not included within the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3:

2>	consider ltm-ReferenceConfiguration in ltm-Config, associated with the cell group for which the LTM cell switch procedure is triggered, to be the current UE configuration for the fields and configurations to be released by the actions above in this procedure;

2>	if measConfig is included within ltm-ReferenceConfiguration in ltm-Config;

3>	perform the measurement configuration procedure as specified in clause 5.5.2 by considering the measConfig within ltm-ReferenceConfiguration in ltm-Config as the received measConfig:

NOTE 1:	When the UE considers the reference configuration to be the current UE configuration, the UE should store fields and configurations that are part of the reference configuration but should not execute any actions or procedures triggered by the reception of an RRCReconfiguration message which are described in clause 5.3.5.3, unless specified otherwise in this clause.

1>	if the LTM cell switch is triggered by an indication from lower layers:

2>	apply the RRCReconfiguration message in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config identified by the LTM candidate configuration identity received from lower layers according to clause 5.3.5.3;

1>	else (LTM cell switch triggered upon cell selection performed while timer T311 was running):

2>	apply the RRCReconfiguration message in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config related to the LTM candidate configuration identity for the selected cell (i.e., in accordance with 5.3.7.3) according to clause 5.3.5.3;

1>	if the LTM cell switch is triggered on the MCG:

2>	release the radio bearer(s) using the master key and the MCG logical channel(s) that were part of the UE configuration before this LTM cell switch procedure but not part of the LTM candidate configuration either indicated by lower layers or for the selected cell in accordance with 5.3.7.3, or the LTM reference configuration (in case the LTM candidate configuration does not include ltm-ConfigComplete);

1>	else, if the LTM cell switch is triggered on the SCG:

2>	release the radio bearer(s) using the secondary key and the SCG logical channel(s) that were part of the UE configuration before this LTM cell switch procedure but not part of the LTM candidate configuration either indicated by lower layers or for the selected cell in accordance with 5.3.7.3, or the LTM reference configuration (in case the LTM candidate configuration does not include ltm-ConfigComplete);

NOTE 2:	When ltm-ConfigComplete is not included for an LTM candidate configuration, before an LTM cell switch is triggered a UE implementation may generate and store an RRCReconfiguration message by applying the received LTM candidate configuration on top of the LTM reference configuration, and the stored RRCReconfiguration message is applied when the LTM cell switch is triggered. It is up to the UE to ensure that the RRC reconfiguration applied at the time of LTM cell switch is in accordance with the latest LTM reference configuration and LTM candidate configuration.

[TS 38.321, clause 5.18.35]

The network may instruct the UE to perform LTM cell switch procedure by sending the LTM Cell Switch Command MAC CE described in clause 6.1.3.75.

The MAC entity shall:

1>	if the MAC entity receives an LTM Cell Switch Command MAC CE on a Serving Cell:

2>	indicate to upper layers that the LTM cell switch procedure is triggered and the Target Configuration ID included in the LTM Cell Switch Command MAC CE;

2>	if the MAC reset operation as specified in clause 5.12 is performed, as requested by upper layers:

3>	if Timing Advance Command value (hexa-decimal) is not set as FFF:

4>	process the received Timing Advance Command (see clause 5.2);

4>	consider the RACH-less LTM cell switch to be ongoing;

4>	if the MAC entity is associated with the SCG:

5>	indicate to upper layers to skip the Random Access procedure for this LTM cell switch.

3>	else if the UE is configured with UE-based Timing Advance measurement as specified in TS 38.331 [5] and the UE has successfully measured the Timing Advance for the SpCell of the indicated LTM target configuration:

4>	process the measured Timing Advance (see clause 5.2);

4>	consider the RACH-less LTM cell switch to be ongoing.

4>	if the MAC entity is associated with the SCG:

5>	indicate to upper layers to skip the Random Access procedure for this LTM cell switch.

3>	indicate to lower layers the information regarding the TCI state information included in the LTM Cell Switch Command MAC CE.

[TS 38.323, clause 5.3]

When the successful delivery of a PDCP SDU is confirmed by PDCP status report, the transmitting PDCP entity shall discard the PDCP SDU along with the corresponding PDCP Data PDU.

When the discardTimer or discardTimerForLowImportance expires for a PDCP SDU, the transmitting PDCP entity shall:

-	if pdu-SetDiscard is configured:

-	discard all PDCP SDUs belonging to the PDU Set to which the PDCP SDU belongs along with the corresponding PDCP Data PDUs;

NOTE 1:	PDCP SDUs subsequently received from upper layers are also discarded if they belong to the PDU Set.

-	else:

-	discard the PDCP SDU along with the corresponding PDCP Data PDU.

If the corresponding PDCP Data PDU has already been submitted to lower layers, the discard is indicated to lower layers.

For SRBs, when upper layers request a PDCP SDU discard, the PDCP entity shall discard all stored PDCP SDUs and PDCP PDUs.

NOTE 2:	Discarding a PDCP SDU already associated with a PDCP SN causes a SN gap in the transmitted PDCP Data PDUs, which increases PDCP reordering delay in the receiving PDCP entity. It is up to UE implementation how to minimize SN gap after SDU discard.

[TS 38.323, clause 7.1]

This clause describes the state variables used in PDCP entities in order to specify the PDCP protocol. The state variables defined in this clause are normative.

All state variables are non-negative integers, and take values from 0 to [232 – 1].

PDCP Data PDUs are numbered integer sequence numbers (SN) cycling through the field: 0 to [2[pdcp-SN-SizeUL] – 1] or 0 to [2[pdcp-SN-SizeDL] – 1] or 0 to [2[sl-PDCP-SN-Size] – 1].

The transmitting PDCP entity shall maintain the following state variables:

a)	TX_NEXT

This state variable indicates the COUNT value of the next PDCP SDU to be transmitted. The initial value is 0, except for SRBs configured with state variables continuation. For target SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding source SRB. For source SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding target SRB.

The receiving PDCP entity shall maintain the following state variables:

a)	RX_NEXT

This state variable indicates the COUNT value of the next PDCP SDU expected to be received. The initial value is 0, except for sidelink broadcast and groupcast, for SRBs configured with state variables continuation, for multicast MRBs whose PDCP COUNT is not synchronized as indicated by upper layer, and for broadcast MRBs. For NR sidelink communication for broadcast and groupcast or sidelink SRB4 for NR sidelink discovery, the initial value of the SN part of RX_NEXT is (x +1) modulo (2[sl-PDCP-SN-Size]), where x is the SN of the first received PDCP Data PDU. For multicast MRBs whose PDCP COUNT is not synchronized as indicated by upper layer, and for broadcast MRBs, the initial value of the SN part of RX_NEXT is (x +1) modulo (2[PDCP-SN-SizeDL]), where x is the SN of the first received PDCP Data PDU. For target SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding source SRB. For source SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding target SRB.

NOTE 1:	For NR sidelink communication for broadcast and groupcast or sidelink SRB4 for NR sidelink discovery, it is up to UE implementation to select the HFN part for RX_NEXT such that initial value of RX_DELIV should be a positive value.

NOTE 2:	For multicast MRBs whose PDCP COUNT is not synchronized as indicated by upper layer, and for broadcast MRBs, the initial value of the HFN part of RX_NEXT is set by UE implementation.

b)	RX_DELIV

This state variable indicates the COUNT value of the first PDCP SDU not delivered to the upper layers, but still waited for. The initial value is 0, except for sidelink broadcast and groupcast, for SRBs configured with state variables continuation, and for MRBs. For NR sidelink communication for broadcast and groupcast or sidelink SRB4 for NR sidelink discovery, the initial value of the SN part of RX_DELIV is (x – 0.5 × 2[sl-PDCP-SN-Size–1]) modulo (2[sl-PDCP-SN-Size]), where x is the SN of the first received PDCP Data PDU. For multicast MRBs whose PDCP COUNT is not synchronized as indicated by upper layer, and for broadcast MRBs, the initial value of the SN part of RX_DELIV is set to (x – 0.5 × 2[PDCP-SN-SizeDL–1]) modulo (2[PDCP-SN-SizeDL]), where x is the SN of the first received PDCP Data PDU. For multicast MRBs, the initial value of RX_DELIV is set, if provided, by initialRX-DELIV in TS 38.331 [3]. For target SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding source SRB. For source SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding target SRB.

NOTE 3:	For multicast MRBs whose PDCP COUNT is not synchronized as indicated by upper layer, and for broadcast MRBs, the initial value of the HFN part of RX_DELIV is set by UE implementation.

c)	RX_REORD

This state variable indicates the COUNT value following the COUNT value associated with the PDCP Data PDU which triggered t-Reordering. For target SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding source SRB. For source SRB configured with state variables continuation, the initial value is the value stored in PDCP entity for the corresponding target SRB.

7.1.3.5.14.3	Test description

7.1.3.5.14.3.1	Pre-test conditions

System Simulator:

Same Pre-test conditions as in clause 7.1.3.0 except the following:

-	The UE is in state NR RRC_CONNECTED using generic procedure parameter Connectivity (NR-DC).

-	NR Cell 1 is the PCell and NR Cell 10 is the PSCell and NRCell 3 is Neighbour Cell of the PCell.

-	System information combination NR-4 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used in NR cells.

-	Test Loop Function (On) with UE test loop mode A (message condition UE TEST LOOP MODE A to return one UL PDCP SDU per DL PDCP SDU) activated according to TS 38.508-1 [4], table 4.5.4.2-3.

-	PDCP is configured for 12 bit SN

Table 7.1.3.5.14.3.1-1: PDCP Settings

| Parameter | Value |
| --- | --- |
| Discard_Timer | 1500 ms |

7.1.3.5.14.3.2	Test procedure sequence

Table 7.1.3.5.14.3.2-1 and 7.1.3.5.14.3.2-2 illustrates the downlink power levels to be applied for NR Cell 1, NR Cell 10 and NR Cell 3 at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while the configuration marked "T1" and "T2", are applied at the point indicated in the Main behaviour description in Table 7.1.3.5.14.3.2-3.

Table 7.1.3.5.14.3.2-1: Time instances of cell power level and parameter changes for conducted test environment

|  | Parameter | Unit | NR Cell 1 | NR Cell 10 | NR Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -82 | -85 | -91 | Power levels are such that entry condition for event A3 is not satisfied for the neighbour NR cell 3 Mn + Ofn + Ocn – Hys > Ms + Ofs + Ocs + Off |
| T1 | SS/PBCH SSS EPRE | dBm/ SCS | -82 | off | -79 | Power levels are such that entry condition for event A3 is satisfied for inter-frequency neighbour NR cell 3 Mn + Ofn + Ocn – Hys > Ms + Ofs + Ocs + Off |

Table 7.1.3.5.14.3.2-2: Time instances of cell power level and parameter changes for OTA test environment

|  | Parameter | Unit | NR Cell 1 | NR Cell 10 | NR Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -82 | -91 | -100 | Power levels are such that entry condition for event A3 is not satisfied for the neighbour NR cell 3 Mn + Ofn + Ocn – Hys > Mp + Ofp + Ocp + Off |
| T1 | SS/PBCH SSS EPRE | dBm/ SCS | -100 | off | -82 | Power levels are such that entry condition for event A3 is satisfied for inter-frequency neighbour NR cell 3 Mn + Ofn + Ocn – Hys > Ms + Ofs + Ocs + Off |

Table 7.1.3.5.14.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message including MeasConfig to setup NR measurement and reporting for inter-frequency event A3 on NR Cell 1. | <-- | RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message. | --> | RRCReconfigurationComplete | - | - |
| 3 | SS re-adjusts the cell-specific reference signal level according to row "T1" in table 7.1.3.5.14.3.2-2 | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message to report event A3 with the measured RSRP value for NR Cell 3 in NR Cell 1. | --> | MeasurementReport | - | - |
| 5 | The SS transmits an RRCReconfiguration message with LTM-Candidate-r18 for NR Cell 3 on NR Cell 1. | <-- | RRCReconfiguration | - | - |
| 6 | The UE transmits RRCReconfigurationComplete message. | --> | RRCReconfigurationComplete | - | - |
| 7 | The SS creates 5 PDCP Data PDUs and the PDCP SN = "0" within TX_NEXT. |  | - | - | - |
| - | EXCEPTION: Step 8 shall be repeated for k=0 to 2 (increment=1) with the below specified PDU size sent to the UE: Data PDU#1 = 46 bytes for k=0 Data PDU#2 = 62 bytes for k=1 Data PDU#3 = 78 bytes for k=2 | - | - | - | - |
| 8 | The SS sends a PDCP Data PDU via RLC-UM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = k After having sent a PDU, the SS sets PDCP SN is set to k+1 within TX_NEXT. | <-- | PDCP DATA PDU (SN=k) | - | - |
| 9 | The SS transmits the candidate cell TCI activation MAC CE with Candidate Cell ID set to 0 for NR Cell 3 on NR Cell 1. | <-- | MAC PDU (Candidate Cell TCI Activation/Deactivation MAC CE) | - | - |
| 10 | The SS transmits an LTM Cell Switch Command MAC CE with Timing Advance Command value set to 0 and MCG of the Target Configuration ID value set to 0 on NR Cell 1 to the UE. | <-- | MAC PDU (LTM Cell Switch Command MAC CE) | - | - |
| 11 | Wait for Discard_Timer to expire. Note: According to TS38.508-1, timer tolerance should be 10% of Discard_Timer. | - | - | - | - |
| - | EXCEPTION: Step 12 shall be repeated for k=3 to 4 (increment=1) with the below specified PDU size sent to the UE: Data PDU#4 = 94 bytes for k=3 Data PDU#5 = 110 bytes for k=4 | - | - | - | - |
| 12 | The SS sends a PDCP Data PDU via RLC-UM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = k After having sent a PDU, the SS sets PDCP SN is set to k+1 within TX_NEXT. | <-- | PDCP DATA PDU (SN=k) | - | - |
| - | EXCEPTION: Step 16 may happen anytime after Step 13 | - | - | - | - |
| 13 | The SS starts normal UL grant allocation on NR Cell3. | - | - | - | - |
| 14 | Check: Does UE transmit a PDCP Data PDU # 4 of size 94 bytes on NR Cell 3? (Note1) (Note 2) | --> | PDCP Data PDU # 4 | 1 | P |
| 15 | Check: Does UE transmit a PDCP Data PDU # 5 of size 110 bytes on NR Cell 3? (Note1) (Note 2) | --> | PDCP Data PDU # 5 | 1 | P |
| 16 | Check: Does the UE transmit RRCReconfigurationComplete message on NR Cell 3? | --> | RRCReconfigurationComplete | 1 | P |
| Note 1: PDCP Data PDU contents are checked to verify that the UL PDU is same as the DL PDU. According to the Note in TS 38.323 [19] clause 5.3 in case of PDCP SDUs being discarded it is up to the UE implementation which SN to be used and therefore the SN cannot be checked. Note 2: PDCP PDUs at steps 14 and 15 may be received by the SS in the same slot or in multiple slots (max one MAC PDU in a slot). |  |  |  |  |  |

7.1.3.5.14.3.3	Specific message contents

Table 7.1.3.5.14.3.3-1: SchedulingRequestConfig (Preamble)

| Derivation Path: 38.508-1 [4], Table 4.6.3-155 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| sr-TransMax | n64 |  |  |

Table 7.1.3.5.14.3.3-2: RRCReconfiguration (step 1, Table 7.1.3.5.14.3.2-3)

Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 with condition NR_MEAS

Table 7.1.3.5.14.3.3-3: MeasConfig (step 1, Table 7.1.3.5.14.3.2-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR | Table 7.1.3.5.14.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-EventA3 | Table 7.1.3.5.14.3.3-5 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.5.14.3.3-4: MeasObjectNR (Table 7.1.3.5.14.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for SSB of NR Cell 1 |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| nrofSS-BlocksToAverage | Not present |  |  |
| } |  |  |  |

Table 7.1.3.5.14.3.3-5: ReportConfigNR-EventA3 (Table 7.1.3.5.14.3.3-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-142 with condition EVENT_A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | 2 | 1 dB(2*0.5 dB) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportAmount | r1 |  |  |
| maxNrofRS-IndexesToReport | 1 |  |  |
| includeBeamMeasurements | true |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | true |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.5.14.3.3-6: MeasurementReport (step 4, Table 7.1.3.5.14.3.2-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-5A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| measurementReport SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServingMOList SEQUENCE (SIZE (1..maxNrofServingCells)) OF MeasResultServMO { | 1 entry |  |  |
| MeasResultServMO[1] SEQUENCE { |  | entry 1 |  |
| servCellId | ServCellIndex of NR Cell 1 |  | Step 4 |
| measResultServingCell SEQUENCE { |  |  |  |
| physCellId | PCI of NR Cell 1 |  | Step 4 |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not present |  |  |
|  | Not checked |  | pc_ss_SINR_Meas |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListNR SEQUENCE (SIZE (1.. maxCellReport)) OF MeasResultNR { | 1 entry |  |  |
| MeasResultNR[1] SEQUENCE { |  | entry 1 |  |
| physCellId | Physical layer cell identity of NR Cell 3 |  | Step 4 |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | Not present |  |  |
| sinr | Not present |  |  |
| } |  |  |  |
| resultsCSI-RS-Cell | Not present |  |  |
| } |  |  |  |
| rsIndexResults | Not present |  |  |
| } |  |  |  |
| cgi-Info | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.5.14.3.3-7: RRCReconfiguration (step 5, Table 7.1.3.5.14.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 with condition NR-DC and LTM |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ltm-Config-r18 CHOICE { |  |  |  |
| setup | LTM-Config-r18 | Table 7.1.3.5.14.3.3-8 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.5.14.3.3-8: LTM-Config-r18 (Table 7.1.3.5.14.3.3-7)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-67D |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-Config-r18 ::= SEQUENCE { |  |  |  |
| ltm-CandidateToAddModList-r18 SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-Candidate-r18 { | 1 entry |  |  |
| LTM-Candidate-r18[1] | LTM-Candidate-r18 | Table 7.1.3.5.14.3.3-9 |  |
| } |  |  |  |
| ltm-CSI-ResourceConfig-r18 | Not present |  |  |
| } |  |  |  |

Table 7.1.3.5.14.3.3-9: LTM-Candidate-r18 (Table 7.1.3.5.14.3.3-8)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-67C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-Candidate-r18 ::= SEQUENCE { |  |  |  |
| ltm-CandidateId-r18 | 1 |  |  |
| ltm-CandidatePCI-r18 | Physical layer cell identity of NR Cell 3 |  |  |
| ltm-CandidateConfig-r18 OCTET STRING (CONTAINING RRCReconfiguration) { |  |  |  |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig specified in TS 38.508-1 [4] Table 4.6.3-19 with condition PCell_change |  |  |
| masterKeyUpdate ::= SEQUENCE { |  |  |  |
| keySetChangeIndicator | false |  |  |
| nextHopChainingCount | 0 |  |  |
| nas-Container | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ltm-TCI-Info-r18 | LTM-TCI-Info specified in TS 38.508-1 [4] Table 4.6.3-67I with condition Joint |  |  |
| } |  |  |  |

##### 7.1.3.5.14A.1	LTM cell switch / PCell / Master Key / PDCP persists / Intra-band Contiguous CA

7.1.3.5.14A.1.1	Test Purpose (TP)

(1)

with { UE in RRC_Connected state with candidate configuration for LTM PCell and the SRB/DRB are using the master key }

ensure that {

when { UE receives Random Access Resources in LTM Cell Switch Command MAC CE }

then { UE keeps the associated PDCP and SDAP entities, their state variables, buffers and timers }

}

7.1.3.5.14A.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331 clauses 5.3.5.18.6 and 5.3.1.2, TS 38.323 clauses 5.2.2.1, 5.2.2.2 and 5.2.2.3. Unless otherwise stated these are Rel-18 requirements.

[TS 38.331, clause 5.3.5.18.6]

Upon the indication by lower layers that an LTM cell switch procedure is triggered, or upon performing LTM cell switch following cell selection performed while timer T311 was running, as specified in 5.3.7.3, the UE shall:

1>	if the LTM cell switch is triggered on the MCG:

2>	release/clear all current dedicated and common radio configurations which have neither been received via SRB1 within mrdc-SecondaryCellGroup, nor via SRB3 except for the following:

-	the radio bearer configuration (configured via RadioBearerConfig)

-	the logicalChannelIdentity and logicalChannelIdentityExt of RLC bearers configured in RLC-BearerConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the bh-LogicalChannelIdentity of BH RLC channels configured in BH-RLC-ChannelConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the UE variables VarLTM-ServingCellNoResetID and VarLTM-ServingCellUE-MeasuredTA-ID;

-	the ltm-Config;

-	the MCG C-RNTI;

-	the AS security configurations associated with the master key;

-	the logged measurement configuration;

1>	else, if the LTM cell switch is triggered on the SCG:

2>	release/clear all current dedicated and common radio configurations which have been received either via SRB1 within mrdc-SecondaryCellGroup, or via SRB3 except for the following:

-	the radio bearer configuration (configured via RadioBearerConfig IE)

-	the logicalChannelIdentity and logicalChannelIdentityExt of RLC bearers configured in RLC-BearerConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the bh-LogicalChannelIdentity of BH RLC channels configured in BH-RLC-ChannelConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the UE variables VarLTM-ServingCellNoResetID and VarLTM-ServingCellUE-MeasuredTA-ID;

-	the ltm-Config;

-	the AS security configurations associated with the secondary key;

1>	for each SRB/DRB in the current UE configuration:

2>	if the LTM cell switch is triggered on the MCG and the SRB/DRB using the master key; or

2>	if the LTM cell switch is triggered on the SCG and the SRB/DRB using the secondary key:

3>	keep the associated PDCP and SDAP entities, their state variables, buffers and timers;

3>	release all fields related to the SRB/DRB configuration except for srb-Identity and drb-Identity;

3>	apply the default SRB configuration defined in 9.2.1 for the corresponding SRB;

NOTE 00:	For all radio bearers and RLC bearers included in the LTM candidate configuration to be applied at an LTM cell switch execution (i.e., as derived from the LTM reference configuration and the LTM candidate configuration), even if those radio bearers and RLC bearers were configured before the LTM cell switch execution, the network includes fields as specified for the initial setup of radio bearers and RLC bearers and sets the values previously stored by the UE for the fields that cannot be modified according to presence conditions or field descriptions.

[TS 38.331, clause 5.3.1.2]

For a UE provided with an sk-counter, keyToUse indicates whether the UE uses the master key (KgNB) or the secondary key (S-KeNB or S-KgNB) for a particular DRB. The secondary key is derived from the master key and sk-Counter, as defined in TS 33.501[11]. Whenever there is a need to refresh the secondary key, e.g. upon change of MN with KgNB change or to avoid COUNT reuse, the security key update is used (see 5.3.5.7). When the UE is in NR-DC, the network may provide a UE configured with an SCG with an sk-Counter even when no DRB is setup using the secondary key (S-KgNB) in order to allow the configuration of SRB3. The network can also provide the UE with an sk-Counter, even if no SCG is configured, when using SN terminated MCG bearers.

[TS 38.323, clause 5.2.2.1]

In this clause, following definitions are used:

-	HFN(State Variable): the HFN part (i.e. the number of most significant bits equal to HFN length) of the State Variable;

-	SN(State Variable): the SN part (i.e. the number of least significant bits equal to PDCP SN length) of the State Variable;

-	RCVD_SN: the PDCP SN of the received PDCP Data PDU, included in the PDU header;

-	RCVD_HFN: the HFN of the received PDCP Data PDU, calculated by the receiving PDCP entity;

-	RCVD_COUNT: the COUNT of the received PDCP Data PDU = [RCVD_HFN, RCVD_SN].

At reception of a PDCP Data PDU from lower layers, the receiving PDCP entity shall determine the COUNT value of the received PDCP Data PDU, i.e. RCVD_COUNT, as follows:

-	if RCVD_SN < SN(RX_DELIV) – Window_Size:

-	RCVD_HFN = HFN(RX_DELIV) + 1.

-	else if RCVD_SN >= SN(RX_DELIV) + Window_Size:

-	RCVD_HFN = HFN(RX_DELIV) – 1.

-	else:

-	RCVD_HFN = HFN(RX_DELIV);

-	RCVD_COUNT = [RCVD_HFN, RCVD_SN].

After determining the COUNT value of the received PDCP Data PDU = RCVD_COUNT, the receiving PDCP entity shall:

-	perform deciphering and integrity verification of the PDCP Data PDU using COUNT = RCVD_COUNT;

-	if integrity verification fails:

-	indicate the integrity verification failure to upper layer;

-	discard the PDCP Data PDU and consider it as not received;

-	if RCVD_COUNT < RX_DELIV; or

-	if the PDCP Data PDU with COUNT = RCVD_COUNT has been received before:

-	discard the PDCP Data PDU;

If the received PDCP Data PDU with COUNT value = RCVD_COUNT is not discarded above, the receiving PDCP entity shall:

-	store the resulting PDCP SDU in the reception buffer;

-	if RCVD_COUNT >= RX_NEXT:

-	update RX_NEXT to RCVD_COUNT + 1.

-	if outOfOrderDelivery is configured:

-	deliver the resulting PDCP SDU to upper layers after performing header decompression using EHC.

-	if RCVD_COUNT = RX_DELIV:

-	deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before;

-	all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from COUNT = RX_DELIV, where consecutively associated COUNT value(s) include COUNT value(s) of both the stored PDCP SDU(s) and PDCP SDU(s) which are considered as discarded as specified in clause 5.16.2;

-	update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers and is not considered as discarded as specified in clause 5.16.2, with COUNT value > RX_DELIV;

-	if t-Reordering is running, and if RX_DELIV >= RX_REORD:

-	stop and reset t-Reordering.

-	if t-Reordering is not running (includes the case when t-Reordering is stopped due to actions above), and RX_DELIV < RX_NEXT:

-	update RX_REORD to RX_NEXT;

-	start t-Reordering.

[TS 38.323, clause 5.2.2.2]

When t-Reordering expires, the receiving PDCP entity shall:

-	deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before:

-	all stored PDCP SDU(s) with associated COUNT value(s) < RX_REORD;

-	all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from RX_REORD, where consecutively associated COUNT value(s) include COUNT value(s) of both the stored PDCP SDU(s) and PDCP SDU(s) which are considered as discarded as specified in clause 5.16.2;

-	update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers and is not considered as discarded as specified in clause 5.16.2, with COUNT value >= RX_REORD;

-	if RX_DELIV < RX_NEXT:

-	update RX_REORD to RX_NEXT;

-	start t-Reordering.

[TS 38.323, clause 5.2.2.3]

When the value of the t-Reordering is reconfigured by upper layers while the t-Reordering is running, the receiving PDCP entity shall:

-	update RX_REORD to RX_NEXT;

-	stop and restart t-Reordering.

7.1.3.5.14A.1.3	Test description

7.1.3.5.14A.1.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.3.0 with the exception except the following:

-	3 NR cells (NR Cell 1, NR Cell 3 and NR Cell 6) are configured. NR Cell 1 is the PCell, NR Cell 3 is the SCell. NR Cell 6 is the neighbor cell of PCell.

-	The UE uses the master key for SRB and DRB.

-	AM DRB is configured according to Table 7.1.3.5.14A.1.3.1-1.

Table 7.1.3.5.14A.1.3.1-1: PDCP parameters

| Parameter | Value |
| --- | --- |
| t-Reordering | ms750 |
| statusReportRequired | Not present |

7.1.3.5.14A.1.3.2	Test procedure sequence

Tables 7.1.3.5.14A.1.3.2-1 and 7.1.3.5.14A.1.3.2-2 illustrate the downlink power levels to be applied for NR Cell 1, NR Cell 3 and NR Cell 6 at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while the configuration marked "T1" is applied at the point indicated in the Main behaviour description in Table 7.1.3.5.14A.1.3.2-3.

Table 7.1.3.5.14A.1.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 | NR Cell 6 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -85 | -88 | -94 | Power levels are such that entry condition for event A3 is not satisfied for the neighbour NR cell |
| T1 | SS/PBCH SSS EPRE | dBm/ SCS | -85 | -88 | -82 | Power levels are such that entry condition for event A3 is satisfied for intra-frequency neighbour NR cell |

Table 7.1.3.5.14A.1.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 | NR Cell 6 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -88 | -91 | -100 | Power levels are such that entry condition for event A3 is not satisfied for the neighbour NR cell |
| T1 | SS/PBCH SSS EPRE | dBm/ SCS | -88 | -91 | -82 | Power levels are such that entry condition for event A3 is satisfied for intra-frequency neighbour NR cell |

Table 7.1.3.5.14A.1.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message including MeasConfig to setup NR measurement and reporting for event A3 on NR Cell 1. | <-- | RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message. | --> | RRCReconfigurationComplete | - | - |
| 3 | The SS sends the PDCP SDU #1 D/C field = 1 (PDCP Data PDU) and PDCP SN = 1 on NR Cell 1. | <-- | (PDCP SDU #1) | - | - |
| 4 | The SS sends the PDCP SDU #2 D/C field = 1 (PDCP Data PDU) and PDCP SN =2 on NR Cell 1. | <-- | (PDCP SDU #2) | - | - |
| 5 | SS re-adjusts the cell-specific reference signal level according to row "T1" in table 7.1.3.5.14A.1.3.2-1/2. | - | - | - | - |
| 6 | The UE transmits a MeasurementReport message to report event A3 with the measured RSRP for NR Cell 6 on NR Cell 1. | --> | MeasurementReport | - | - |
| 7 | The SS transmits an RRCReconfiguration message with LTM-Candidate-r18 for NR Cell 6 on NR Cell 1. | <-- | RRCReconfiguration | - | - |
| 8 | The UE transmits an RRCReconfigurationComplete message. | --> | RRCReconfigurationComplete | - | - |
| 9 | The SS transmits the candidate cell TCI activation MAC CE with Candidate Cell ID set to 0 on NR Cell 1. | <-- | MAC PDU (Candidate Cell TCI state activation/deactivation MAC CE) | - | - |
| 10 | The SS transmits DCI 1_0 PDCCH order to trigger the UE PRACH transmission on NR Cell 1. | <-- | (PDCCH Order) | - | - |
| 11 | The UE transmits the PRACH preamble on NR Cell 6. | --> | (PRACH Preamble) | - | - |
| 12 | The SS transmits LTM cell switch command MAC CE with Timing Advance Command value set to 0 and Target Configuration ID value set to 0 to the UE on NR Cell 1. | <-- | MAC PDU (LTM Cell Switch Command MAC CE) | - | - |
| 13 | The UE transmits an RRCReconfigurationComplete message on NR Cell 6. | --> | RRCReconfigurationComplete | - | - |
| 14 | The SS sends the PDCP SDU #0 D/C field = 1 (PDCP Data PDU) and PDCP SN = 0 on NR Cell 6. | <-- | (PDCP SDU #0) | - | - |
| 15 | Check: Does the UE transmit the PDCP SDU #0 via the AM RLC entity D/C field = 1 (PDCP Data PDU) and PDCP SN = 0 on NR Cell 6? (Note 1) | --> | (PDCP SDU #0) | 1 | P |
| 16 | Check: Does the UE transmit the PDCP SDU #1 via the AM RLC entity D/C field = 1 (PDCP Data PDU) and PDCP SN = 1 on NR Cell 6? (Note 1) | --> | (PDCP SDU #1) | 1 | P |
| 17 | Check: Does the UE transmit the PDCP SDU #2 via the AM RLC entity D/C field = 1 (PDCP Data PDU) and PDCP SN = 2 on NR Cell 6? (Note 1) | --> | (PDCP SDU #2) | 1 | P |
| Note 1: PDCP SDUs at steps 15, 16 and 17 may be received by the SS in the same slot or in multiple slots (max one MAC PDU in a slot). |  |  |  |  |  |

7.1.3.5.14A.1.3.3	Specific message contents

Table 7.1.3.5.14A.1.3.3-1: RRCReconfiguration (step 1, Table 7.1.3.5.14A.1.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with conditions NR_MEAS

Table 7.1.3.5.14A.1.3.3-2: MeasConfig (Table 7.1.3.5.14A.1.3.3-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR | Table 7.1.3.5.14A.1.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-EventA3 | Table 7.1.3.5.14A.1.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.5.14A.1.3.3-3: MeasObjectNR (Table 7.1.3.5.14A.1.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR ::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for SSB of NR Cell 6 |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| nrofSS-BlocksToAverage | Not present |  |  |
| } |  |  |  |

Table 7.1.3.5.14A.1.3.3-4: ReportConfigNR-EventA3 (Table 7.1.3.5.14A.1.3.3-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-142 with condition EVENT_A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | 2 | 1 dB(2*0.5 dB) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportAmount | infinity |  |  |
| maxNrofRS-IndexesToReport | 1 |  |  |
| includeBeamMeasurements | true |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | true |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.5.14A.1.3.3-5: MeasurementReport (step 6, Table 7.1.3.5.14A.1.3.2-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-5A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| measurementReport SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServingMOList SEQUENCE (SIZE (1..maxNrofServingCells)) OF MeasResultServMO { | 1 entry |  |  |
| MeasResultServMO[1] SEQUENCE { |  | entry 1 |  |
| servCellId | ServCellIndex of NR Cell 1 |  |  |
| measResultServingCell SEQUENCE { |  |  |  |
| physCellId | PCI of NR Cell 1 |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not present |  |  |
|  | Not checked |  | pc_ss_SINR_Meas |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListNR SEQUENCE (SIZE (1.. maxCellReport)) OF MeasResultNR { | 1 entry |  |  |
| MeasResultNR[1] SEQUENCE { |  | entry 1 |  |
| physCellId | Physical layer cell identity of NR Cell 6 |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | Not present |  |  |
| sinr | Not present |  |  |
| } |  |  |  |
| resultsCSI-RS-Cell | Not present |  |  |
| } |  |  |  |
| rsIndexResults | Not present |  |  |
| } |  |  |  |
| cgi-Info | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.5.14A.1.3.3-6: RRCReconfiguration (step 7, Table 7.1.3.5.14A.1.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with conditions LTM

Table 7.1.3.5.14A.1.3.3-7: LTM-Config-r18(Table 7.1.3.5.14A.1.3.3-6)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-67D |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-Config-r18 ::= SEQUENCE { |  |  |  |
| ltm-CandidateToAddModList-r18 SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-Candidate-r18 { | 1 entry |  |  |
| LTM-Candidate-r18[1] | LTM-Candidate-r18 | Table 7.1.3.5.14A.1.3.3-8 |  |
| } |  |  |  |
| ltm-CSI-ResourceConfig-r18 | Not present |  |  |
| } |  |  |  |

Table 7.1.3.5.14A.1.3.3-8: LTM-Candidate-r18 (Table 7.1.3.5.14A.1.3.3-7)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-67C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-Candidate-r18 ::= SEQUENCE { |  |  |  |
| ltm-CandidateId-r18 | 1 |  |  |
| ltm-CandidatePCI-r18 | Physical layer cell identity of NR Cell 6 |  |  |
| ltm-CandidateConfig-r18 OCTET STRING (CONTAINING RRCReconfiguration) { |  |  |  |
| RRCReconfiguration SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig specified in TS 38.508-1 [4] Table 4.6.3-19 with condition PCell_change |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ltm-TCI-Info-r18 | LTM-TCI-Info specified in TS 38.508-1 [4] Table 4.6.3-67I with condition Joint |  |  |
| } |  |  |  |

##### 7.1.3.5.14A.2	LTM cell switch / PCell / Master Key / PDCP persists / Inter-band CA

The scope and description of the present TC is the same as test case 7.1.3.5.14A.1 with the following differences:

-	CA configuration: Inter-band CA replaces Intra-band Contiguous CA

-	Cells configuration: NR Cell 10 replaces NR Cell 3

##### 7.1.3.5.14A.3	LTM cell switch / PCell / Master Key / PDCP persists / Intra-band non-Contiguous CA

The scope and description of the present TC is the same as test case 7.1.3.5.14A.1 with the following differences:

-	CA configuration: Intra-band non-Contiguous CA replaces Intra-band Contiguous CA.

##### 7.1.3.5.15	LTM cell switch / SCG / Secondary Key / PDCP persists

7.1.3.5.15.1	Test Purpose (TP)

(1)

with { UE in RRC_Connected state with candidate configuration for LTM SCG and the SRB/DRB are using the secondary key }

ensure that {

when { UE receives Random Access Resources in LTM Cell Switch Command MAC CE }

then { UE keeps the associated PDCP and SDAP entities, their state variables, buffers and timers }

}

7.1.3.5.15.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331 clauses 5.3.5.18.6 and 5.3.1.2, TS 38.323, clause 5.2.2.1, 5.2.2.2 and 5.2.2.3. Unless otherwise stated these are Rel-18 requirements.

[TS 38.331, clause 5.3.5.18.6]

Upon the indication by lower layers that an LTM cell switch procedure is triggered, or upon performing LTM cell switch following cell selection performed while timer T311 was running, as specified in 5.3.7.3, the UE shall:

1>	if the LTM cell switch is triggered on the MCG:

2>	release/clear all current dedicated and common radio configurations which have neither been received via SRB1 within mrdc-SecondaryCellGroup, nor via SRB3 except for the following:

-	the radio bearer configuration (configured via RadioBearerConfig)

-	the logicalChannelIdentity and logicalChannelIdentityExt of RLC bearers configured in RLC-BearerConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the bh-LogicalChannelIdentity of BH RLC channels configured in BH-RLC-ChannelConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the UE variables VarLTM-ServingCellNoResetID and VarLTM-ServingCellUE-MeasuredTA-ID;

-	the ltm-Config;

-	the MCG C-RNTI;

-	the AS security configurations associated with the master key;

-	the logged measurement configuration;

1>	else, if the LTM cell switch is triggered on the SCG:

2>	release/clear all current dedicated and common radio configurations which have been received either via SRB1 within mrdc-SecondaryCellGroup, or via SRB3 except for the following:

-	the radio bearer configuration (configured via RadioBearerConfig IE)

-	the logicalChannelIdentity and logicalChannelIdentityExt of RLC bearers configured in RLC-BearerConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the bh-LogicalChannelIdentity of BH RLC channels configured in BH-RLC-ChannelConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the UE variables VarLTM-ServingCellNoResetID and VarLTM-ServingCellUE-MeasuredTA-ID;

-	the ltm-Config;

-	the AS security configurations associated with the secondary key;

1>	for each SRB/DRB in the current UE configuration:

2>	if the LTM cell switch is triggered on the MCG and the SRB/DRB using the master key; or

2>	if the LTM cell switch is triggered on the SCG and the SRB/DRB using the secondary key:

3>	keep the associated PDCP and SDAP entities, their state variables, buffers and timers;

3>	release all fields related to the SRB/DRB configuration except for srb-Identity and drb-Identity;

3>	apply the default SRB configuration defined in 9.2.1 for the corresponding SRB;

NOTE 00:	For all radio bearers and RLC bearers included in the LTM candidate configuration to be applied at an LTM cell switch execution (i.e., as derived from the LTM reference configuration and the LTM candidate configuration), even if those radio bearers and RLC bearers were configured before the LTM cell switch execution, the network includes fields as specified for the initial setup of radio bearers and RLC bearers and sets the values previously stored by the UE for the fields that cannot be modified according to presence conditions or field descriptions.

1>	apply the default L1 parameter values as specified in corresponding physical layer specifications except for the parameters for which values are provided in SIB1;

1>	use the default values specified in 9.2.3 for timers T310, T311 and constants N310, N311 associated with the cell group for which the LTM cell switch procedure is triggered, where T310, N310, and N311 are for both MCG and SCG, and T311 is only for the MCG;

1>	apply the default MAC Cell Group configuration as specified in 9.2.2 for the cell group for which the LTM cell switch procedure is triggered;

1>	if the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 does not contain the field ltm-NoResetID and if the UE does not have any value stored of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID; or

1>	if the value of field ltm-NoResetID contained within the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID:

2>	for each logicalChannelIdentity and logicalChannelIdentityExt that is part of the current UE configuration for the cell group for which the LTM cell switch procedure is triggered:

3>	if servedRadioBearer is set to drb-Identity:

4>	after the end of this procedure, re-establish the corresponding RLC entity as specified in TS 38.322 [4], after applying the LTM configuration in ltm-CandidateConfig within the LTM-Candidate IE in ltm-Config;

2>	for each bh-LogicalChannelIdentity that is part of the current UE configuration for the cell group for which the LTM cell switch procedure is triggered:

3>	after the end of this procedure, re-establish the corresponding RLC entity as specified in TS 38.322 [4], after applying the LTM configuration in ltm-CandidateConfig within the LTM-Candidate IE in ltm-Config;

2>	for each drb-Identity value that is part of the current UE configuration:

3>	if this DRB is an AM DRB:

4>	after the end of this procedure, trigger the PDCP entity of this DRB to perform data recovery as specified in TS 38.323 [5], after applying the LTM configuration in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config;

2>	if the value of field ltm-NoResetID contained within the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID:

3>	replace the value of ltm-ServingCellNoResetID in VarLTM-ServingCellNoResetID with the value of ltm-NoResetID in the LTM-Candidate in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3;

1> if the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 contains the field ltm-UE-MeasuredTA-ID:

2>	if the value of ltm-UE-MeasuredTA-ID is not equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID:

3>	replace the value of ltm-ServingCellUE-MeasuredTA-ID in VarLTM-ServingCellUE-MeasuredTA-ID with the value received within ltm-UE-MeasuredTA-ID;

3>	for each LTM-Candidate IE in ltm-Config:

4>	if the value of ltm-UE-MeasuredTA-ID within LTM-Candidate IE is equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID:

5>	inform lower layers that the UE is configured with UE-based TA measurements for the LTM-Candidate;

4>	else:

5>	inform lower layers that the UE is not configured with UE-based TA measurements for the LTM-Candidate;

NOTE 0:	The UE is not expected to perform UE-based TA measurements for an SpCell.

1>	if ltm-ConfigComplete is not included within the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3:

2>	consider ltm-ReferenceConfiguration in ltm-Config, associated with the cell group for which the LTM cell switch procedure is triggered, to be the current UE configuration for the fields and configurations to be released by the actions above in this procedure;

2>	if measConfig is included within ltm-ReferenceConfiguration in ltm-Config;

3>	perform the measurement configuration procedure as specified in clause 5.5.2 by considering the measConfig within ltm-ReferenceConfiguration in ltm-Config as the received measConfig:

NOTE 1:	When the UE considers the reference configuration to be the current UE configuration, the UE should store fields and configurations that are part of the reference configuration but should not execute any actions or procedures triggered by the reception of an RRCReconfiguration message which are described in clause 5.3.5.3, unless specified otherwise in this clause.

1>	if the LTM cell switch is triggered by an indication from lower layers:

2>	apply the RRCReconfiguration message in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config identified by the LTM candidate configuration identity received from lower layers according to clause 5.3.5.3;

1>	else (LTM cell switch triggered upon cell selection performed while timer T311 was running):

2>	apply the RRCReconfiguration message in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config related to the LTM candidate configuration identity for the selected cell (i.e., in accordance with 5.3.7.3) according to clause 5.3.5.3;

1>	if the LTM cell switch is triggered on the MCG:

2>	release the radio bearer(s) using the master key and the MCG logical channel(s) that were part of the UE configuration before this LTM cell switch procedure but not part of the LTM candidate configuration either indicated by lower layers or for the selected cell in accordance with 5.3.7.3, or the LTM reference configuration (in case the LTM candidate configuration does not include ltm-ConfigComplete);

1>	else, if the LTM cell switch is triggered on the SCG:

2>	release the radio bearer(s) using the secondary key and the SCG logical channel(s) that were part of the UE configuration before this LTM cell switch procedure but not part of the LTM candidate configuration either indicated by lower layers or for the selected cell in accordance with 5.3.7.3, or the LTM reference configuration (in case the LTM candidate configuration does not include ltm-ConfigComplete);

NOTE 2:	When ltm-ConfigComplete is not included for an LTM candidate configuration, before an LTM cell switch is triggered a UE implementation may generate and store an RRCReconfiguration message by applying the received LTM candidate configuration on top of the LTM reference configuration, and the stored RRCReconfiguration message is applied when the LTM cell switch is triggered. It is up to the UE to ensure that the RRC reconfiguration applied at the time of LTM cell switch is in accordance with the latest LTM reference configuration and LTM candidate configuration.

[TS 38.331, clause 5.3.1.2]

For a UE provided with an sk-counter, keyToUse indicates whether the UE uses the master key (KgNB) or the secondary key (S-KeNB or S-KgNB) for a particular DRB. The secondary key is derived from the master key and sk-Counter, as defined in TS 33.501[11]. Whenever there is a need to refresh the secondary key, e.g. upon change of MN with KgNB change or to avoid COUNT reuse, the security key update is used (see 5.3.5.7). When the UE is in NR-DC, the network may provide a UE configured with an SCG with an sk-Counter even when no DRB is setup using the secondary key (S-KgNB) in order to allow the configuration of SRB3. The network can also provide the UE with an sk-Counter, even if no SCG is configured, when using SN terminated MCG bearers.

[TS 38.323, clause 5.2.2.1]

In this clause, following definitions are used:

-	HFN(State Variable): the HFN part (i.e. the number of most significant bits equal to HFN length) of the State Variable;

-	SN(State Variable): the SN part (i.e. the number of least significant bits equal to PDCP SN length) of the State Variable;

-	RCVD_SN: the PDCP SN of the received PDCP Data PDU, included in the PDU header;

-	RCVD_HFN: the HFN of the received PDCP Data PDU, calculated by the receiving PDCP entity;

-	RCVD_COUNT: the COUNT of the received PDCP Data PDU = [RCVD_HFN, RCVD_SN].

At reception of a PDCP Data PDU from lower layers, the receiving PDCP entity shall determine the COUNT value of the received PDCP Data PDU, i.e. RCVD_COUNT, as follows:

-	if RCVD_SN < SN(RX_DELIV) – Window_Size:

-	RCVD_HFN = HFN(RX_DELIV) + 1.

-	else if RCVD_SN >= SN(RX_DELIV) + Window_Size:

-	RCVD_HFN = HFN(RX_DELIV) – 1.

-	else:

-	RCVD_HFN = HFN(RX_DELIV);

-	RCVD_COUNT = [RCVD_HFN, RCVD_SN].

After determining the COUNT value of the received PDCP Data PDU = RCVD_COUNT, the receiving PDCP entity shall:

-	perform deciphering and integrity verification of the PDCP Data PDU using COUNT = RCVD_COUNT;

-	if integrity verification fails:

-	indicate the integrity verification failure to upper layer;

-	discard the PDCP Data PDU and consider it as not received;

-	if RCVD_COUNT < RX_DELIV; or

-	if the PDCP Data PDU with COUNT = RCVD_COUNT has been received before:

-	discard the PDCP Data PDU;

If the received PDCP Data PDU with COUNT value = RCVD_COUNT is not discarded above, the receiving PDCP entity shall:

-	store the resulting PDCP SDU in the reception buffer;

-	if RCVD_COUNT >= RX_NEXT:

-	update RX_NEXT to RCVD_COUNT + 1.

-	if outOfOrderDelivery is configured:

-	deliver the resulting PDCP SDU to upper layers after performing header decompression using EHC.

-	if RCVD_COUNT = RX_DELIV:

-	deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before;

-	all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from COUNT = RX_DELIV, where consecutively associated COUNT value(s) include COUNT value(s) of both the stored PDCP SDU(s) and PDCP SDU(s) which are considered as discarded as specified in clause 5.16.2;

-	update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers and is not considered as discarded as specified in clause 5.16.2, with COUNT value > RX_DELIV;

-	if t-Reordering is running, and if RX_DELIV >= RX_REORD:

-	stop and reset t-Reordering.

-	if t-Reordering is not running (includes the case when t-Reordering is stopped due to actions above), and RX_DELIV < RX_NEXT:

-	update RX_REORD to RX_NEXT;

-	start t-Reordering.

[TS 38.323, clause 5.2.2.2]

When t-Reordering expires, the receiving PDCP entity shall:

-	deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before:

-	all stored PDCP SDU(s) with associated COUNT value(s) < RX_REORD;

-	all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from RX_REORD, where consecutively associated COUNT value(s) include COUNT value(s) of both the stored PDCP SDU(s) and PDCP SDU(s) which are considered as discarded as specified in clause 5.16.2;

-	update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers and is not considered as discarded as specified in clause 5.16.2, with COUNT value >= RX_REORD;

-	if RX_DELIV < RX_NEXT:

-	update RX_REORD to RX_NEXT;

-	start t-Reordering.

[TS 38.323, clause 5.2.2.3]

When the value of the t-Reordering is reconfigured by upper layers while the t-Reordering is running, the receiving PDCP entity shall:

-	update RX_REORD to RX_NEXT;

-	stop and restart t-Reordering.

7.1.3.5.15.3	Test description

7.1.3.5.15.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.3.0 with the exception except the following:

-	3 NR cells (NR Cell 1, NR Cell 10 and NR Cell 30) are configured. NR Cell 1 is the PCell, NR Cell 10 is the PSCell, neighbour cell NR Cell 30 is on the same frequency as the PSCell.

-	The UE is in state RRC_CONNECTED using generic procedure parameter Connectivity (NR-DC), Bearers (MCG(s) and SCG) established according to TS 38.508-1 [4], clause 4.5.4. The UE uses the secondary key for SCG DRB.

-	AM DRB is configured according to Table 7.1.3.5.15.3.1-1.

Table 7.1.3.5.15.3.1-1: PDCP parameters

| Parameter | Value |
| --- | --- |
| t-Reordering | ms750 |
| statusReportRequired | Not present |

7.1.3.5.15.3.2	Test procedure sequence

Tables 7.1.3.5.15.3.2-1 and 7.1.3.5.15.3.2-2 illustrate the downlink power levels to be applied for NR Cell 1, NR Cell 10 and NR Cell 30 at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while the configuration marked "T1" is applied at the point indicated in the Main behaviour description in Table 7.1.3.5.15.3.2-3.

Table 7.1.3.5.15.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 10 | NR Cell 30 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -85 | -88 | -94 | Power levels are such that entry condition for event A3 is not satisfied for the neighbour NR cell |
| T1 | SS/PBCH SSS EPRE | dBm/ SCS | -85 | -88 | -82 | Power levels are such that entry condition for event A3 is satisfied for intra-frequency neighbour NR cell |

Table 7.1.3.5.15.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 10 | NR Cell 30 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -88 | -91 | -100 | Power levels are such that entry condition for event A3 is not satisfied for the neighbour NR cell |
| T1 | SS/PBCH SSS EPRE | dBm/ SCS | -88 | -91 | -82 | Power levels are such that entry condition for event A3 is satisfied for intra-frequency neighbour NR cell |

Table 7.1.3.5.15.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message on NR Cell 1 containing an RRCReconfiguration message including MeasConfig to setup NR measurement and reporting for intra-frequency event A3. | <-- | RRCReconfiguration (RRCReconfiguration) | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message containing RRCReconfigurationComplete message on NR Cell 1. | --> | RRCReconfigurationComplete (RRCReconfigurationComplete) | - | - |
| 3 | The SS sends the PDCP SDU #1 D/C field = 1 (PDCP Data PDU) and PDCP SN = 1 on NR Cell 10. | <-- | (PDCP SDU #1) | - | - |
| 4 | The SS sends the PDCP SDU #2 D/C field = 1 (PDCP Data PDU) and PDCP SN =2 on NR Cell 10. | <-- | (PDCP SDU #2) | - | - |
| 5 | SS re-adjusts the cell-specific reference signal level according to row "T1" in table 7.1.3.5.15.3.2-1/2. | - | - | - | - |
| 6 | The UE transmits a MeasurementReport message embedded in NR RRC message ULInformationTransferMRDC on NR Cell 1 to report event A3 with the measured RSRP forNR Cell 30. |  | ULInformationTransferMRDC (MeasurementReport) | - | - |
| 7 | The SS transmits an RRCReconfiguration message on NR Cell 1 containing an RRCReconfiguration message with LTM-Candidate-r18 for NR Cell 30. |  | RRCReconfiguration (RRCReconfiguration) | - | - |
| 8 | The UE transmits an RRCReconfigurationComplete message containing RRCReconfigurationComplete message on NR Cell 1. | --> | RRCReconfigurationComplete (RRCReconfigurationComplete) | - | - |
| 9 | The SS transmits the candidate cell TCI activation MAC CE with Candidate Cell ID set to 0 for NR Cell 30 on NR Cell 10. | <-- | MAC PDU (Candidate Cell TCI state activation/deactivation MAC CE) | - | - |
| 10 | The SS transmits DCI 1_0 PDCCH order to trigger the UE PRACH transmission on NR Cell 10. | <-- | (PDCCH Order) | - | - |
| 11 | The UE transmits the PRACH preamble on NR Cell 30. | --> | (PRACH Preamble) | - | - |
| 12 | The SS transmits LTM cell switch command MAC CE with Timing Advance Command value set to 0 and Target Configuration ID value set to 0 on NR Cell 10 to the UE. | <-- | MAC PDU (LTM Cell Switch Command MAC CE) | - | - |
| 13 | The UE transmits an ULInformationTransferMRDC message containing RRCReconfigurationComplete message on NR Cell 1. | --> | ULInformationTransferMRDC (RRCReconfigurationComplete) | - | - |
| 14 | The SS sends the PDCP SDU #0 D/C field = 1 (PDCP Data PDU) and PDCP SN = 0 on NR Cell 30. | <-- | (PDCP SDU #0) | - | - |
| 15 | Check: Does the UE transmit the PDCP SDU #0 via the AM RLC entity D/C field = 1 (PDCP Data PDU) and PDCP SN = 0 on NR Cell 30 in 1 second? (Note 1) | --> | (PDCP SDU #0) | 1 | P |
| 16 | Check: Does the UE transmit the PDCP SDU #1 via the AM RLC entity D/C field = 1 (PDCP Data PDU) and PDCP SN = 1 on NR Cell 30 in 1 second? (Note 1) | --> | (PDCP SDU #1) | 1 | P |
| 17 | Check: Does the UE transmit the PDCP SDU #2 via the AM RLC entity D/C field = 1 (PDCP Data PDU) and PDCP SN = 2 on NR Cell 30 in 1 second? (Note 1) | --> | (PDCP SDU #2) | 1 | P |
| Note 1: PDCP SDUs at steps 15, 16 and 17 may be received by the SS in the same slot or in multiple slots (max one MAC PDU in a slot). |  |  |  |  |  |

7.1.3.5.15.3.3	Specific message contents

Table 7.1.3.5.15.3.3-1: RRCReconfiguration (Preamble, Table 7.1.3.5.15.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| mrdc-SecondaryCellGroupConfig CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| mrdc-SecondaryCellGroup CHOICE { |  |  |  |
| nr-SCG | OCTET STRING (CONTAINING RRCReconfiguration) as defined in TS 38.508-1 [4], Table 4.6.1-13 with condition NR-DC_SCG |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| radioBearerConfig2 | OCTET STRING (CONTAINING RadioBearerConfig) as defined in TS 38.508-1 [4], with condition DRBn and SecondaryKeys |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.5.15.3.3-2: RRCReconfiguration (step 1, Table 7.1.3.5.15.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with conditions NR_MEAS and NR-DC

Table 7.1.3.5.15.3.3-3: MeasConfig (Table 7.1.3.5.15.3.3-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR | Table 7.1.3.5.15.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-EventA3 | Table 7.1.3.5.15.3.3-5 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.5.15.3.3-4: MeasObjectNR (Table 7.1.3.5.15.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR ::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for SSB of NR Cell 10 |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| nrofSS-BlocksToAverage | Not present |  |  |
| } |  |  |  |

Table 7.1.3.5.15.3.3-5: ReportConfigNR-EventA3 (Table 7.1.3.5.15.3.3-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-142 with condition EVENT_A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | 2 | 1 dB(2*0.5 dB) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportAmount | infinity |  |  |
| maxNrofRS-IndexesToReport | 1 |  |  |
| includeBeamMeasurements | true |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | true |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.5.15.3.3-6: MeasurementReport (step 6, Table 7.1.3.5.15.3.2-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-5A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| measurementReport SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServingMOList SEQUENCE (SIZE (1..maxNrofServingCells)) OF MeasResultServMO { | 1 entry |  |  |
| MeasResultServMO[1] SEQUENCE { |  | entry 1 |  |
| servCellId | ServCellIndex of NR Cell 10 |  |  |
| measResultServingCell SEQUENCE { |  |  |  |
| physCellId | PCI of NR Cell 10 |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not present |  |  |
|  | Not checked |  | pc_ss_SINR_Meas |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListNR SEQUENCE (SIZE (1.. maxCellReport)) OF MeasResultNR { | 1 entry |  |  |
| MeasResultNR[1] SEQUENCE { |  | entry 1 |  |
| physCellId | Physical layer cell identity of NR Cell 30 |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | Not present |  |  |
| sinr | Not present |  |  |
| } |  |  |  |
| resultsCSI-RS-Cell | Not present |  |  |
| } |  |  |  |
| rsIndexResults | Not present |  |  |
| } |  |  |  |
| cgi-Info | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.5.15.3.3-7: RRCReconfiguration (step 7, Table 7.1.3.5.15.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| mrdc-SecondaryCellGroupConfig CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| mrdc-SecondaryCellGroup CHOICE { |  |  |  |
| nr-SCG | RRCReconfiguration | Table 7.1.3.5.15.3.3-8 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.5.15.3.3-8: RRCReconfiguration (step 7, Table 7.1.3.5.15.3.3-7)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ltm-Config-r18 CHOICE { |  |  |  |
| setup | LTM-Config-r18 | Table 7.1.3.5.15.3.3-9 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.5.15.3.3-9: LTM-Config-r18(Table 7.1.3.5.15.3.3-8)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-67D |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-Config-r18 ::= SEQUENCE { |  |  |  |
| ltm-CandidateToAddModList-r18 SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-Candidate-r18 { | 1 entry |  |  |
| LTM-Candidate-r18[1] | LTM-Candidate-r18 | Table 7.1.3.5.15.3.3-9 |  |
| } |  |  |  |
| ltm-CSI-ResourceConfig-r18 | Not present |  |  |
| } |  |  |  |

Table 7.1.3.5.15.3.3-10: LTM-Candidate-r18 (Table 7.1.3.5.15.3.3-9)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-67C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-Candidate-r18 ::= SEQUENCE { |  |  |  |
| ltm-CandidateId-r18 | 1 |  |  |
| ltm-CandidatePCI-r18 | Physical layer cell identity of NR Cell 30 |  |  |
| ltm-CandidateConfig-r18 OCTET STRING (CONTAINING RRCReconfiguration) { | RRCReconfiguration | Table 7.1.3.5.15.3.3-11 |  |
| ltm-TCI-Info-r18 | LTM-TCI-Info specified in TS 38.508-1 [4] Table 4.6.3-67I with condition Joint |  |  |
| } |  |  |  |

Table 7.1.3.5.15.3.3-11: RRCReconfiguration (Table 7.1.3.5.15.3.3-10)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| secondaryCellGroup | CellGroupConfig specified in TS 38.508-1 [4] Table 4.6.3-19 with condition NR-DC_SCG |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 7.1.3.6	PDCP UDC

##### 7.1.3.6.1	PDCP UDC / No dictionary

7.1.3.6.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with PDCP UDC configured }

ensure that {

when { pre-defined dictionary is not configured }

then { UE set the compression buffer to all zeros }

}

(2)

with { UE in RRC_CONNECTED state with PDCP UDC configured }

ensure that {

when { UE transmits a PDCP Data SDU on a DRB with UDC configuration }

then { UE sets FU to 1 and compresses the data for each transmitted PDU and calculates the checksum correctly}

}

7.1.3.6.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.323, clauses 5.14.2, annex B.1, B.2.1 and B.2.2. Unless otherwise stated these are Rel-17 requirements.

[TS 38.323, clause 5.14.2]

The PDCP entities associated with DRBs can be configured by upper layers, see TS 38.331 [3], to use UDC. If UDC is configured, the UE shall apply UDC compression function (details see Annex B) to process the received PDCP SDU from upper layers corresponding to the configured DRB. The size of compression buffer is configured by upper layers via bufferSize. If pre-defined dictionary is configured by upper layers, the UE shall first set the compression buffer to all zeros and then prefill the configured pre-defined dictionary in the compression buffer upon configuration of UDC. If pre-defined dictionary is not configured by upper layers, UE shall set the compression buffer to all zeros.

[TS 38.323, annex B.1]

A UDC packet consists of a UDC header and a UDC data block. A UDC data block contains either DEFLATE compressed blocks generated by UDC protocol or original PDCP SDU for SDU not compressed by UDC protocol; the type is specified in FU field (details see Annex B.2.2.1) in UDC header. The FR field (details see Annex B.2.2.2) and the Checksum field (details see Annex B.2.2.3) in UDC header are used only if FU field is set to 1.

If reset procedure is triggered, after performing the reset, the FR field in UDC header of the first compressed PDU shall be set to 1.

NOTE:	UE is allowed not to compress the PDCP SDUs if the UL data rate before compression is higher than what the UE is capable of.

[TS 38.323, annex B.2.1]

Figure B.2.1-1 shows the format of UDC Header and UDC Data Block.

Figure B.2.1-1: UDC header and UDC data block format

[TS 38.323, annex B.2.2.1]

Length: 1 bit

Indication of whether this packet is compressed by UDC protocol or not. Value '1' means the packet is compressed by UDC protocol.

Table B.2.2.1-1: FU field

| Bit | Description |
| --- | --- |
| 0 | Packet is not compressed using UDC protocol |
| 1 | Packet is compressed using UDC protocol |

[TS 38.323, annex B.2.2.2]

Length: 1 bit

Indication of whether UDC compression buffer is reset or not. Value '1' means this is the first compressed packet after UDC buffer reset.

Table B.2.2.2-1: FR field

| Bit | Description |
| --- | --- |
| 0 | Compression buffer is not reset. |
| 1 | Compression buffer has been reset. |

[TS 38.323, annex B.2.2.3]

Length: 4 bits

This field contains the validation bits for the compression buffer content: The checksum is calculated by the content of current compression buffer before the current packet is put into buffer.

The checksum is derived from the values of the first 4 bytes and the last 4 bytes in the whole compression buffer. The calculation is described as follows:

-	Each byte is divided into two 4-bit numbers.

-	The 16 4-bit numbers are added together to obtain a sum;

-	The checksum is one's complement of the right-most 4 bits (i.e. 4 LSB) of the sum.

An example of checksum calculation is shown in Annex B.2.3.

7.1.3.6.1.3	Test description

7.1.3.6.1.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.3.0 with UDC configuration.

7.1.3.6.1.3.2	Test procedure sequence

Table 7.1.3.6.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS creates 5 PDCP Data PDUs and the Next_PDCP_TX_SN is set to "0". | - | - | - | - |
| 2 | The SS sends the PDCP Data PDU#0 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 0. | <-- | PDCP PDU | - | - |
| 3 | Check: Does UE send the PDCP Data PDU#0 via RLC-AM RB with the following content: D/C field = 1 (PDCP Data PDU) , PDCP SN = 0, FU = 1, FR = 0, Checksum = 1111? Data is previously received data from PDU #0 after decompression. (Note 1) | --> | PDCP PDU | 1, 2 | P |
| 4 | The SS sends the PDCP Data PDU#1 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 1. | <-- | PDCP PDU | - | - |
| 5 | Check: Does UE send the PDCP Data PDU#1 via RLC-AM RB with the following content: D/C field = 1 (PDCP Data PDU) , PDCP SN = 1, FU = 1, FR = 0, the value of Checksum is checked (Note 2)? Data is previously received data from PDU #1 after decompression. (Note 1) | --> | PDCP PDU | 2 | P |
| 6 | The SS sends the PDCP Data PDU#2 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 2. | <-- | PDCP PDU | - | - |
| 7 | Check: Does UE send the PDCP Data PDU#2 via RLC-AM RB with the following content: D/C field = 1 (PDCP Data PDU) , PDCP SN = 2, FU = 1, FR = 0, the value of Checksum is checked (Note 2)? Data is previously received data from PDU #2 after decompression. (Note 1) | --> | PDCP PDU | 2 | P |
| 8 | The SS sends the PDCP Data PDU#3 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 3. | <-- | PDCP PDU | - | - |
| 9 | Check: Does UE send the PDCP Data PDU#3 via RLC-AM RB with the following content: D/C field = 1 (PDCP Data PDU) , PDCP SN = 3, FU = 1, FR = 0, the value of Checksum is checked (Note 2). Data is previously received data from PDU #3 after decompression. (Note 1) | --> | PDCP PDU | 2 | P |
| 10 | The SS sends the PDCP Data PDU#4 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 4. | <-- | PDCP PDU | - | - |
| 11 | Check: Does UE send the PDCP Data PDU#4 via RLC-AM RB with the following content: D/C field = 1 (PDCP Data PDU) , PDCP SN = 4, FU = 1, FR = 0, the value of Checksum is checked (Note 2)? Data is previously received data from PDU #4 after decompression. (Note 1) | --> | PDCP PDU | 2 | P |
| Note 1: The SS acknowledges the received data. Note 2: The SS compares the received value of Checksum and calculated value of Checksum based on the compression buffer. |  |  |  |  |  |

7.1.3.6.1.3.3	Specific message contents

None.

##### 7.1.3.6.2	PDCP UDC / Pre-defined dictionary

7.1.3.6.2.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with PDCP UDC configured }

ensure that {

when { pre-defined dictionary is configured }

then { UE prefill the configured pre-defined dictionary in the compression buffer }

}

(2)

with { UE in RRC_CONNECTED state with PDCP UDC configured }

ensure that {

when { UE transmits a PDCP Data SDU on a DRB with UDC configuration }

then { UE sets FU to 1 and compresses the data for each transmitted PDU and calculates the checksum correctly}

}

7.1.3.6.2.2	Conformance requirements

Same as conformance requirements in clause 7.1.3.6.1.2

7.1.3.6.2.3	Test description

7.1.3.6.2.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.3.0 with UDC configuration exception of PDCP parameters according to Table 7.1.3.6.2.3.1-1.

Table 7.1.3.6.2.3.1-1: PDCP parameters

| PDCP-Config uplinkDataCompression-r17 setup newSetup dictionary-r17 | sip-SDP |
| --- | --- |

7.1.3.6.2.3.2	Test procedure sequence

Table 7.1.3.6.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS creates 5 PDCP Data PDUs and the Next_PDCP_TX_SN is set to "0". | - | - | - | - |
| 2 | The SS sends the PDCP Data PDU#0 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 0. | <-- | PDCP PDU | - | - |
| 3 | Check: Does UE send the PDCP Data PDU#0 via RLC-AM RB with the following content: D/C field = 1 (PDCP Data PDU) , PDCP SN = 0, FU = 1, FR = 0, the value of Checksum is calculated on pre-defined dictionary? Data is previously received data from PDU #0 after decompression. (Note 1) | --> | PDCP PDU | 1, 2 | P |
| 4 | The SS sends the PDCP Data PDU#1 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 1. | <-- | PDCP PDU | - | - |
| 5 | Check: Does UE send the PDCP Data PDU#1 via RLC-AM RB with the following content: D/C field = 1 (PDCP Data PDU) , PDCP SN = 1, FU = 1, FR = 0, the value of Checksum is checked (Note 2)? Data is previously received data from PDU #1 after decompression. (Note 1) | --> | PDCP PDU | 2 | P |
| 6 | The SS sends the PDCP Data PDU#2 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 2. | <-- | PDCP PDU | - | - |
| 7 | Check: Does UE send the PDCP Data PDU#2 via RLC-AM RB with the following content: D/C field = 1 (PDCP Data PDU) , PDCP SN = 2, FU = 1, FR = 0, the value of Checksum is checked (Note 2)? Data is previously received data from PDU #2 after decompression. (Note 1) | --> | PDCP PDU | 2 | P |
| 8 | The SS sends the PDCP Data PDU#3 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 3. | <-- | PDCP PDU | - | - |
| 9 | Check: Does UE send the PDCP Data PDU#3 via RLC-AM RB with the following content: D/C field = 1 (PDCP Data PDU) , PDCP SN = 3, FU = 1, FR = 0, the value of Checksum is checked (Note 2)? Data is previously received data from PDU #3 after decompression. (Note 1) | --> | PDCP PDU | 2 | P |
| 10 | The SS sends the PDCP Data PDU#4 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 4. | <-- | PDCP PDU | - | - |
| 11 | Check: Does UE send the PDCP Data PDU#4 via RLC-AM RB with the following content: D/C field = 1 (PDCP Data PDU) , PDCP SN = 4, FU = 1, FR = 0, the value of Checksum is checked (Note 2)? Data is previously received data from PDU #4 after decompression. (Note 1) | --> | PDCP PDU | 2 | P |
| Note 1: The SS acknowledges the received data. Note 2: The SS compares the received value of Checksum and calculated value of Checksum based on the compression buffer. |  |  |  |  |  |

7.1.3.6.2.3.3	Specific message contents

None.

##### 7.1.3.6.3	PDCP UDC / checksum error / Reset

7.1.3.6.3.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with PDCP UDC configured }

ensure that {

when { UE receiving the checksum error notification }

then { UE resets the UDC buffer and sets the FR field in UDC header of the first compressed PDU to 1 }

}

(2)

with { UE in RRC_CONNECTED state with PDCP UDC configured }

ensure that {

when { UE transmits a PDCP Data SDU on a DRB with UDC configuration }

then { UE sets FU to 1, and compresses the data for each transmitted PDU and calculates the checksum correctly }

}

7.1.3.6.3.2	Conformance requirements

Same as conformance requirements in clause 7.1.3.6.1.2

7.1.3.6.3.3	Test description

7.1.3.6.3.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.3.0 with UDC configuration.

7.1.3.6.3.3.2	Test procedure sequence

Table 7.1.3.6.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS creates 3 PDCP Data PDUs and the Next_PDCP_TX_SN is set to "0". | - | - | - | - |
| 2 | The SS sends the PDCP Data PDU#0 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 0. | <-- | PDCP PDU | - | - |
| 3 | Check: Does UE send the PDCP Data PDU#0 via RLC-AM RB with the following content: D/C field = 1 (PDCP Data PDU) , PDCP SN = 0, FU = 1, FR = 0, Checksum = 1111? Data is previously received data from PDU #0 after decompression. (Note 1) | --> | PDCP PDU | 2 | P |
| 4 | The SS sends the PDCP Data PDU#1 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 1. | <-- | PDCP PDU | - | - |
| 5 | Check: Does UE send the PDCP Data PDU#1 via RLC-AM RB with the following content: D/C field = 1 (PDCP Data PDU) , PDCP SN = 1, FU = 1, FR = 0, the value of Checksum is checked (Note 2)? Data is previously received data from PDU #1 after decompression. (Note 1) | --> | PDCP PDU | 2 | P |
| 6 | The SS sends the PDCP Control PDU for UDC feedback packet with the following content to the UE: D/C field = 0 (Control PDU) PDU type = 011 (UDC feedback packet) FE = 1 (Checksum Error Notification) | <-- | UDC feedback packet | - | - |
| 7 | The SS sends the PDCP Data PDU#2 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 2. | <-- | PDCP PDU | - | - |
| 8 | Check: Does the UE send the PDCP Data PDU#2 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) , PDCP SN = 2, FU = 1, FR = 1, Checksum = 1111? Data is previously received data from PDU #2 after decompression. (Note 1) | --> | PDCP PDU | 1 | P |
| Note 1: The SS acknowledges the received data. Note 2: The SS compares the received value of Checksum and calculated value of Checksum based on the compression buffer. |  |  |  |  |  |

7.1.3.6.3.3.3	Specific message contents

None.

##### 7.1.3.6.4	PDCP UDC/ Handover/ Intra-frequency

7.1.3.6.4.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with PDCP UDC configured }

ensure that {

when { UE receives an RRCReconfiguration message including a reconfigurationWithSync for Intra-frequency handover and drb-ContinueUDC is not configured }

then { UE maintains its previous UDC configuration for the DRB and resets the UDC buffer and sets the FR field in UDC header of the first compressed PDU to 1 }

}

(2)

with { UE in RRC_CONNECTED state with PDCP UDC configured }

ensure that {

when { UE receives an RRCReconfiguration message including a reconfigurationWithSync for Intra-frequency handover and drb-ContinueUDC is configured }

then { UE maintains its previous UDC configuration for the DRB and continues to use the previous UDC buffer }

}

7.1.3.6.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.323, clauses 5.1.2, TS 38.331, clauses 5.3.5.6.5. Unless otherwise stated these are Rel-17 requirements.

[TS 38.323, clause 5.1.2]

When upper layers request a PDCP entity re-establishment, the UE shall additionally perform once the procedures described in this clause for Uu or PC5 interface. After performing the procedures in this clause, the UE shall follow the procedures in clause 5.2.

When upper layers request a PDCP entity re-establishment, the transmitting PDCP entity shall:

-	for UM DRBs and AM DRBs, reset the ROHC protocol for uplink and start with an IR state in U-mode (as defined in RFC 3095 [8] and RFC 4815 [9]) if drb-ContinueROHC is not configured in TS 38.331 [3];

-	for UM DRBs and AM DRBs, reset the EHC protocol for uplink if drb-ContinueEHC-UL is not configured in TS 38.331 [3];

-	for AM DRBs, reset the UDC compression buffer to all zeros and prefill the dictionary if drb-ContinueUDC is not configured in TS 38.331 [3];

-	for SRBs and UM DRBs, set TX_NEXT to the initial value;

-	for SRBs, discard all stored PDCP SDUs and PDCP PDUs;

-	apply the ciphering algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	apply the integrity protection algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	for UM DRBs, for each PDCP SDU already associated with a PDCP SN but for which a corresponding PDU has not previously been submitted to lower layers, and;

-	for AM DRBs for Uu interface whose PDCP entities were suspended, from the first PDCP SDU for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by lower layers, for each PDCP SDU already associated with a PDCP SN:

-	consider the PDCP SDUs as received from upper layer;

-	perform transmission of the PDCP SDUs in ascending order of the COUNT value associated to the PDCP SDU prior to the PDCP re-establishment without restarting the discardTimer, as specified in clause 5.2.1;

-	for AM DRBs whose PDCP entities were not suspended, from the first PDCP SDU for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by lower layers, perform retransmission or transmission of all the PDCP SDUs already associated with PDCP SNs in ascending order of the COUNT values associated to the PDCP SDU prior to the PDCP entity re-establishment as specified below:

-	perform header compression of the PDCP SDU using ROHC as specified in the clause 5.7.4 and/or using EHC as specified in the clause 5.12.4;

-	If drb-ContinueUDC is configured and if the PDCP SDU has been compressed before:

-	submit the PDCP SDU previously compressed to integrity protection and ciphering function;

-	else:

-	perform uplink data compression of the PDCP SDU as specified in clause 5.14.4, and submit the PDCP SDU to integrity protection and ciphering function;

-	perform integrity protection and ciphering of the PDCP SDU using the COUNT value associated with this PDCP SDU as specified in the clause 5.9 and 5.8;

-	submit the resulting PDCP Data PDU to lower layer, as specified in clause 5.2.1.

[TS 38.331, clause 5.3.5.6.5]

1>	for each drb-Identity value included in the drb-ToAddModList that is part of the current UE configuration and not configured as DAPS bearer:

2>	if the reestablishPDCP is set:

3>	if target RAT of handover is E-UTRA/5GC; or

3>	if the UE is connected to E-UTRA/5GC:

4>	if the UE is capable of E-UTRA/5GC but not capable of NGEN-DC:

5>	if the PDCP entity of this DRB is not configured with cipheringDisabled:

6>	configure the PDCP entity with the ciphering algorithm and KUPenc key configured/derived as specified in TS 36.331 [10], clause 5.4.2.3, i.e. the ciphering configuration shall be applied to all subsequent PDCP PDUs received and sent by the UE;

4>	else (i.e., a UE capable of NGEN-DC):

5>	if the PDCP entity of this DRB is not configured with cipheringDisabled:

6>	configure the PDCP entity with the ciphering algorithm and KUPenc key associated with the master key (KeNB) or the secondary key (S-KgNB), as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent PDCP PDUs received and sent by the UE;

3>	else (i.e., UE connected to NR or UE connected to E-UTRA/EPC (in EN-DC or capable of EN-DC)):

4>	if the PDCP entity of this DRB is not configured with cipheringDisabled:

5>	configure the PDCP entity with the ciphering algorithm and KUPenc key associated with the master key (KeNB/ KgNB) or the secondary key (S-KgNB/S-KeNB), as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent PDCP PDUs received and sent by the UE;

4>	if the PDCP entity of this DRB is configured with integrityProtection:

5>	configure the PDCP entity with the integrity protection algorithms according to securityConfig and apply the KUPint key associated with the master key (KeNB/KgNB) or the secondary key (S-KgNB) as indicated in keyToUse;

3>	if drb-ContinueROHC is included in pdcp-Config:

4>	indicate to lower layer that drb-ContinueROHC is configured;

3>	if drb-ContinueEHC-DL is included in pdcp-Config:

4>	indicate to lower layer that drb-ContinueEHC-DL is configured;

3>	if drb-ContinueEHC-UL is included in pdcp-Config:

4>	indicate to lower layer that drb-ContinueEHC-UL is configured;

3>	if drb-ContinueUDC is included in pdcp-Config:

4>	indicate to lower layer that drb-ContinueUDC is configured;

3>	re-establish the PDCP entity of this DRB as specified in TS 38.323 [5], clause 5.1.2;

7.1.3.6.4.3	Test description

7.1.3.6.4.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.3.0 with UDC configuration except the following:

-	2 NR cells (NR Cell 1 and NR Cell 2) are configured with DRBs in RLC AM mode.

-	The cell power levels are configured as per the Table 7.1.3.6.4.3.1-1.

Table 7.1.3.6.4.3.1-1: Time instances of cell power level in FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | Off |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -82 |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -88 |  |

Table 7.1.3.6.4.3.1-2: Time instances of cell power level in FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -91 | Off |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -91 | -82 |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -82 | 91 |  |

7.1.3.6.4.3.2	Test procedure sequence

Table 7.1.3.6.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS creates 5 PDCP Data PDUs and the Next_PDCP_TX_SN is set to "0". | - | - | - | - |
| 2 | The SS sends the PDCP Data PDU#0 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 0. | <-- | PDCP PDU | - | - |
| 3 | UE sends the PDCP Data PDU#0 via RLC-AM RB on NR Cell 1 with the following content: D/C field = 1 (PDCP Data PDU) , PDCP SN = 0, FU = 1, FR = 0, Checksum = 1111. Data is previously received data from PDU #0 after decompression. (Note 1) | --> | PDCP PDU | - | - |
| 4 | The SS sends the PDCP Data PDU#1 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 1. | <-- | PDCP PDU | - | - |
| 5 | UE sends the PDCP Data PDU#1 via RLC-AM RB on NR Cell 1 with the following content: D/C field = 1 (PDCP Data PDU) , PDCP SN = 1, FU = 1, FR = 0, the value of Checksum is checked (Note 2). Data is previously received data from PDU #1 after decompression. (Note 1) | --> | PDCP PDU | - | - |
| 6 | The SS changes NR Cell 2 parameters according to the row "T1" in table 7.1.3.6.4.3.1-1(FR1) / 7.1.3.6.4.3.1-2(FR2). |  |  |  |  |
| 7 | The SS transmits NR RRCReconfiguration message to perform SpCell change from NR Cell1 to NR Cell 2 | <-- | RRCReconfiguration | - | - |
| 8 | UE transmits an RRCReconfigurationComplete message on NR Cell 2. | --> | RRCReconfigurationComplete | - | - |
| 9 | The SS sends the PDCP Data PDU#2 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 2. | <-- | PDCP PDU | - | - |
| 10 | CHECK: Does UE send the PDCP Data PDU#2 via RLC-AM RB on NR Cell 2 with the following content: D/C field = 1 (PDCP Data PDU) , PDCP SN = 2, FU = 1, FR = 1, Checksum = 1111? Data is previously received data from PDU #2 after decompression. (Note 1) | --> | PDCP PDU | 1 | P |
| 11 | The SS sends the PDCP Data PDU#3 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 3. | <-- | PDCP PDU | - | - |
| 12 | UE sends the PDCP Data PDU#3 via RLC-AM RB on NR Cell 2 with the following content: D/C field = 1 (PDCP Data PDU) , PDCP SN = 3, FU = 1, FR = 0, the value of Checksum is checked (Note 2). Data is previously received data from PDU #3 after decompression. (Note 1) |  | PDCP PDU | - | - |
| 13 | SS adjusts the cell-specific reference signal level according to row "T2". |  |  |  |  |
| 14 | The SS transmits NR RRCReconfiguration message to perform SpCell change from NR Cell 2 to NR Cell 1 | <-- | RRCReconfiguration | - | - |
| 15 | UE transmits an RRCReconfigurationComplete message on NR Cell 1. | --> | RRCReconfigurationComplete | - | - |
| 16 | The SS sends the PDCP Data PDU#4 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 4. | <-- | PDCP PDU | - | - |
| 17 | CHECK: Does UE send the PDCP Data PDU#4 via RLC-AM RB on NR Cell 1 with the following content: D/C field = 1 (PDCP Data PDU) , PDCP SN = 4, FU = 1, FR = 0, the value of Checksum is checked (Note 2)? Data is previously received data from PDU #4 after decompression. (Note 1) | --> | PDCP PDU | 2 | P |
| Note 1: The SS acknowledges the received data. Note 2: The SS compares the received value of Checksum and calculated value of Checksum based on the compression buffer. |  |  |  |  |  |

7.1.3.6.4.3.3	Specific message contents

Table 7.1.3.6.4.3.3-1: RRCReconfiguration (step 7, Table 7.1.3.6.4.3.2-1)

| Derivation Path: 38.508-1 [4], Table: 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | OCTET STRING (CONTAINING CellGroupConfig) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.6.4.3.3-2: RadioBearerConfig (Table 7.1.3.6.4.3.3-1)

| Derivation Path: 38.508-1 [4], Table: 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| drb-Identity | Default DRB of the first PDU session |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.6.4.3.3-3: Void

Table 7.1.3.6.4.3.3-4: CellGroupConfig (Table 7.1.3.6.4.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with condition PCell_change and CFRA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| spCellConfigCommon SEQUENCE { | Same as default ServingCellConfigCommon |  |  |
| physCellId | Physical cell Id of NR Cell 2 |  |  |
| } |  |  |  |
| rach-ConfigDedicated CHOICE { |  |  |  |
| Uplink | RACH-ConfigDedicated |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.6.4.3.3-5: RRCReconfiguration (step 14, Table 7.1.3.6.4.3.2-1)

| Derivation Path: 38.508-1 [4], Table: 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | OCTET STRING (CONTAINING CellGroupConfig) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.6.4.3.3-6: RadioBearerConfig (Table 7.1.3.6.4.3.3-5)

| Derivation Path: 38.508-1 [4], Table: 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| drb-Identity | Default DRB of the first PDU session |  |  |
| pdcp-Config | PDCP-Config |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.6.4.3.3-7: PDCP-Config (Table 7.1.3.6.4.3.3-6)

| Derivation Path: 38.508-1 [4], Table: 4.6.3-99 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-Config ::= SEQUENCE { |  |  |  |
| uplinkDataCompression-r17 CHOICE { |  |  |  |
| setup CHOICE { |  |  |  |
| drb-ContinueUDC | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.6.4.3.3-8: CellGroupConfig (Table 7.1.3.6.4.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with condition PCell_change |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| spCellConfigCommon SEQUENCE { | Same as default ServingCellConfigCommon |  |  |
| physCellId | Physical cell Id of NR Cell 1 |  |  |
| } |  |  |  |
| rach-ConfigDedicated | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 7.1.3.6.5	PDCP UDC/ Handover/ Inter-frequency

7.1.3.6.5.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with PDCP UDC configured }

ensure that {

when { UE receives an RRCReconfiguration message including a reconfigurationWithSync for Inter-frequency handover and drb-ContinueUDC is not configured }

then { UE maintains its previous UDC configuration for the DRB and resets the UDC buffer and sets the FR field in UDC header of the first compressed PDU to 1 }

}

(2)

with { UE in RRC_CONNECTED state with PDCP UDC configured }

ensure that {

when { UE receives an RRCReconfiguration message including a reconfigurationWithSync for Inter-frequency handover and drb-ContinueUDC is configured }

then { UE maintains its previous UDC configuration for the DRB and continues to use the previous UDC buffer }

}

7.1.3.6.5.2	Conformance requirements

Same as conformance requirements in clause 7.1.3.6.4.2

7.1.3.6.5.3	Test description

7.1.3.6.5.3.1	Pre-test conditions

Same as test case 7.1.3.6.4 with the following differences:

-	Cells configuration: NR Cell 3 replaces NR Cell 2.

7.1.3.6.5.3.2	Test procedure sequence

Same as test case 7.1.3.6.4 with the following differences:

-	Cells configuration: NR Cell 3 replaces NR Cell 2.

7.1.3.6.5.3.3	Specific message contents

Same as test case 7.1.3.6.4 with the following differences:

-	Cells configuration: NR Cell 3 replaces NR Cell 2.

##### 7.1.3.6.6	PDCP UDC/ RRC resume

7.1.3.6.6.1	Test Purpose (TP)

(1)

with { UE in RRC_INACTIVE state with PDCP UDC configured }

ensure that {

when { UE receives an RRCResume message and drb-ContinueUDC is not configured }

then { UE maintains its previous UDC configuration for the DRB and resets the UDC buffer and sets the FR field in UDC header of the first compressed PDU to 1 }

}

(2)

with { UE in RRC_INACTIVE state with PDCP UDC configured }

ensure that {

when { UE receives an RRCResume message and drb-ContinueUDC is configured }

then { UE maintains its previous UDC configuration for the DRB and continues to use the previous UDC buffer }

}

7.1.3.6.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.323, clauses 5.1.2, TS 38.331, clauses 5.3.5.6.5, 5.3.8.3, 5.3.13.3. Unless otherwise stated these are Rel-17 requirements.

[TS 38.323, clause 5.1.2]

When upper layers request a PDCP entity re-establishment, the UE shall additionally perform once the procedures described in this clause for Uu or PC5 interface. After performing the procedures in this clause, the UE shall follow the procedures in clause 5.2.

When upper layers request a PDCP entity re-establishment, the transmitting PDCP entity shall:

-	for UM DRBs and AM DRBs, reset the ROHC protocol for uplink and start with an IR state in U-mode (as defined in RFC 3095 [8] and RFC 4815 [9]) if drb-ContinueROHC is not configured in TS 38.331 [3];

-	for UM DRBs and AM DRBs, reset the EHC protocol for uplink if drb-ContinueEHC-UL is not configured in TS 38.331 [3];

-	for AM DRBs, reset the UDC compression buffer to all zeros and prefill the dictionary if drb-ContinueUDC is not configured in TS 38.331 [3];

-	for SRBs and UM DRBs, set TX_NEXT to the initial value;

-	for SRBs, discard all stored PDCP SDUs and PDCP PDUs;

-	apply the ciphering algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	apply the integrity protection algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	for UM DRBs, for each PDCP SDU already associated with a PDCP SN but for which a corresponding PDU has not previously been submitted to lower layers, and;

-	for AM DRBs for Uu interface whose PDCP entities were suspended, from the first PDCP SDU for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by lower layers, for each PDCP SDU already associated with a PDCP SN:

-	consider the PDCP SDUs as received from upper layer;

-	perform transmission of the PDCP SDUs in ascending order of the COUNT value associated to the PDCP SDU prior to the PDCP re-establishment without restarting the discardTimer, as specified in clause 5.2.1;

-	for AM DRBs whose PDCP entities were not suspended, from the first PDCP SDU for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by lower layers, perform retransmission or transmission of all the PDCP SDUs already associated with PDCP SNs in ascending order of the COUNT values associated to the PDCP SDU prior to the PDCP entity re-establishment as specified below:

-	perform header compression of the PDCP SDU using ROHC as specified in the clause 5.7.4 and/or using EHC as specified in the clause 5.12.4;

-	If drb-ContinueUDC is configured and if the PDCP SDU has been compressed before:

-	submit the PDCP SDU previously compressed to integrity protection and ciphering function;

-	else:

-	perform uplink data compression of the PDCP SDU as specified in clause 5.14.4, and submit the PDCP SDU to integrity protection and ciphering function;

-	perform integrity protection and ciphering of the PDCP SDU using the COUNT value associated with this PDCP SDU as specified in the clause 5.9 and 5.8;

-	submit the resulting PDCP Data PDU to lower layer, as specified in clause 5.2.1.

[TS 38.331, clause 5.3.5.6.5]

1>	for each drb-Identity value included in the drb-ToAddModList that is part of the current UE configuration and not configured as DAPS bearer:

2>	if the reestablishPDCP is set:

3>	if target RAT of handover is E-UTRA/5GC; or

3>	if the UE is connected to E-UTRA/5GC:

4>	if the UE is capable of E-UTRA/5GC but not capable of NGEN-DC:

5>	if the PDCP entity of this DRB is not configured with cipheringDisabled:

6>	configure the PDCP entity with the ciphering algorithm and KUPenc key configured/derived as specified in TS 36.331 [10], clause 5.4.2.3, i.e. the ciphering configuration shall be applied to all subsequent PDCP PDUs received and sent by the UE;

4>	else (i.e., a UE capable of NGEN-DC):

5>	if the PDCP entity of this DRB is not configured with cipheringDisabled:

6>	configure the PDCP entity with the ciphering algorithm and KUPenc key associated with the master key (KeNB) or the secondary key (S-KgNB), as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent PDCP PDUs received and sent by the UE;

3>	else (i.e., UE connected to NR or UE connected to E-UTRA/EPC (in EN-DC or capable of EN-DC)):

4>	if the PDCP entity of this DRB is not configured with cipheringDisabled:

5>	configure the PDCP entity with the ciphering algorithm and KUPenc key associated with the master key (KeNB/ KgNB) or the secondary key (S-KgNB/S-KeNB), as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent PDCP PDUs received and sent by the UE;

4>	if the PDCP entity of this DRB is configured with integrityProtection:

5>	configure the PDCP entity with the integrity protection algorithms according to securityConfig and apply the KUPint key associated with the master key (KeNB/KgNB) or the secondary key (S-KgNB) as indicated in keyToUse;

3>	if drb-ContinueROHC is included in pdcp-Config:

4>	indicate to lower layer that drb-ContinueROHC is configured;

3>	if drb-ContinueEHC-DL is included in pdcp-Config:

4>	indicate to lower layer that drb-ContinueEHC-DL is configured;

3>	if drb-ContinueEHC-UL is included in pdcp-Config:

4>	indicate to lower layer that drb-ContinueEHC-UL is configured;

3>	if drb-ContinueUDC is included in pdcp-Config:

4>	indicate to lower layer that drb-ContinueUDC is configured;

3>	re-establish the PDCP entity of this DRB as specified in TS 38.323 [5], clause 5.1.2;

[TS 38.331, clause 5.3.8.3]

1>	if the RRCRelease includes suspendConfig:

2>	reset MAC and release the default MAC Cell Group configuration, if any;

2>	apply the received suspendConfig except the received nextHopChainingCount;

2>	if the sdt-Config is configured:

3>	for each of the DRB in the sdt-DRB-List:

4>	consider the DRB to be configured for SDT;

3>	if sdt-SRB2-Indication is configured:

4>	consider the SRB2 to be configured for SDT;

3>	for each RLC bearer that is not suspended:

4>	re-establish the RLC entity as specified in TS 38.322 [4];

3>	for SRB2 (if it is resumed) and for SRB1:

4>	trigger the PDCP entity to perform SDU discard as specified in TS 38.323 [5];

3>	if sdt-MAC-PHY-CG-Config is configured:

4>	configure the PCell with the configured grant resources for SDT and instruct the MAC entity to start the cg-SDT-TimeAlignmentTimer;

2>	if srs-PosRRC-Inactive is configured:

3>	apply the configuration and instruct MAC to start the inactivePosSRS-TimeAlignmentTimer;

NOTE 1b:	The Network should provide full configuration to UE for SRS for Positioning in RRC_INACTIVE.

2>	remove all the entries within the MCG and the SCG VarConditionalReconfig, if any;

2>	for each measId of the MCG measConfig and for each measId of the SCG measConfig, if configured, if the associated reportConfig has a reportType set to condTriggerConfig:

3>	for the associated reportConfigId:

4>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

3>	if the associated measObjectId is only associated to a reportConfig with reportType set to condTriggerConfig:

4>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

3>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

2>	re-establish RLC entities for SRB1;

2>	if the RRCRelease message with suspendConfig was received in response to an RRCResumeRequest or an RRCResumeRequest1:

3>	stop the timer T319 if running;

3>	in the stored UE Inactive AS context:

4>	replace the KgNB and KRRCint keys with the current KgNB and KRRCint keys;

4>	replace the nextHopChainingCount with the value of nextHopChainingCount received in the RRCRelease message;

4>	replace the cellIdentity with the cellIdentity of the cell the UE has received the RRCRelease message;

4>	if the suspendConfig contains the sl-UEIdentityRemote (i.e. the UE is a L2 U2N Remote UE):

5>	replace the C-RNTI with the value of the sl-UEIdentityRemote;

5>	replace the physical cell identity with the value of the sl-PhysCellId in sl-ServingCellInfo contained in the discovery message received from the connected L2 U2N Relay UE;

4> else:

5>	replace the C-RNTI with the C-RNTI used in the cell (see TS 38.321 [3]) the UE has received the RRCRelease message;

5>	replace the physical cell identity with the physical cell identity of the cell the UE has received the RRCRelease message;

3>	replace the nextHopChainingCount with the value associated with the current KgNB;

3>	stop the timer T319a if running and consider SDT procedure is not ongoing;

2>	else:

3>	store in the UE Inactive AS Context the nextHopChainingCount received in the RRCRelease message, the current KgNB and KRRCint keys, the ROHC state, the EHC context(s), the UDC state, the stored QoS flow to DRB mapping rules, the application layer measurement configuration, the C-RNTI used in the source PCell, the cellIdentity and the physical cell identity of the source PCell, the spCellConfigCommon within ReconfigurationWithSync of the NR PSCell (if configured) and all other parameters configured except for:

-	parameters within ReconfigurationWithSync of the PCell;

-	parameters within ReconfigurationWithSync of the NR PSCell, if configured;

-	parameters within MobilityControlInfoSCG of the E-UTRA PSCell, if configured;

-	servingCellConfigCommonSIB;

-	sl-L2RelayUE-Config, if configured;

-	sl-L2RemoteUE-Config, if configured;

[TS 38.331, clause 5.3.13.3]

The UE shall set the contents of RRCResumeRequest or RRCResumeRequest1 message as follows:

1>	if field useFullResumeID is signalled in SIB1:

2>	select RRCResumeRequest1 as the message to use;

2>	set the resumeIdentity to the stored fullI-RNTI value;

1>	else:

2>	select RRCResumeRequest as the message to use;

2>	set the resumeIdentity to the stored shortI-RNTI value;

1>	restore the RRC configuration, RoHC state, the EHC context(s), the UDC state, the stored QoS flow to DRB mapping rules and the KgNB and KRRCint keys from the stored UE Inactive AS context except for the following:

-	masterCellGroup;

-	mrdc-SecondaryCellGroup, if stored; and

-	pdcp-Config;

7.1.3.6.6.3	Test description

7.1.3.6.6.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.3.0 with UDC configuration.

7.1.3.6.6.3.2	Test procedure sequence

Table 7.1.3.6.6.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS creates 5 PDCP Data PDUs and the Next_PDCP_TX_SN is set to "0". | - | - | - | - |
| 2 | The SS sends the PDCP Data PDU#0 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 0. | <-- | PDCP PDU | - | - |
| 3 | UE sends the PDCP Data PDU#0 via RLC-AM RB on NR Cell 1 with the following content: D/C field = 1 (PDCP Data PDU) , PDCP SN = 0, FU = 1, FR = 0, Checksum = 1111. Data is previously received data from PDU #0 after decompression. (Note 1) | --> | PDCP PDU | - | - |
| 4 | The SS sends the PDCP Data PDU#1 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 1. | <-- | PDCP PDU | - | - |
| 5 | UE sends the PDCP Data PDU#1 via RLC-AM RB on NR Cell 1 with the following content: D/C field = 1 (PDCP Data PDU) , PDCP SN = 1, FU = 1, FR = 0, the value of Checksum is checked (Note 2). Data is previously received data from PDU #1 after decompression. (Note 1) | --> | PDCP PDU | - | - |
| 6 | The SS transmits an RRCRelease message with suspendConfig | <-- | NR RRC: RRCRelease | - | - |
| 7 | The SS transmits a Paging message including a matched identities (correct fullI-RNTI). | <-- | NR RRC: Paging | - | - |
| 8 | The UE transmits an RRCResumeRequest message to resume RRC Connection by setting resumeIdentity to the stored shortI-RNTI value. | --> | NR RRC: RRCResumeRequest | - | - |
| 9 | The SS transmits an RRCResume message without uplinkDataCompression. | <-- | NR RRC: RRCResume | - | - |
| 10 | The UE transmits an RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 11 | The SS sends the PDCP Data PDU#2 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 2. | <-- | PDCP PDU | - | - |
| 12 | CHECK: Does UE send the PDCP Data PDU#2 via RLC-AM RB with the following content: D/C field = 1 (PDCP Data PDU) , PDCP SN = 2, FU = 1, FR = 1, Checksum = 1111? Data is previously received data from PDU #2 after decompression. (Note 1) | --> | PDCP PDU | 1 | P |
| 13 | The SS sends the PDCP Data PDU#3 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 3. | <-- | PDCP PDU | - | - |
| 14 | UE sends the PDCP Data PDU#3 via RLC-AM RB with the following content: D/C field = 1 (PDCP Data PDU) , PDCP SN = 3, FU = 1, FR = 0, the value of Checksum is checked (Note 2). Data is previously received data from PDU #3 after decompression. (Note 1) | --> | PDCP PDU | - | - |
| 15 | The SS transmits an RRCRelease message with suspendConfig | <-- | NR RRC: RRCRelease | - | - |
| 16 | The SS transmits a Paging message including a matched identities (correct fullI-RNTI). | <-- | NR RRC: Paging | - | - |
| 17 | The UE transmits an RRCResumeRequest message to resume RRC Connection by setting resumeIdentity to the stored shortI-RNTI value. | --> | NR RRC: RRCResumeRequest | - | - |
| 18 | The SS transmits an RRCResume message with drb-ContinueUDC. | <-- | NR RRC: RRCResume | - | - |
| 19 | The UE transmits an RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 20 | The SS sends the PDCP Data PDU#4 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 4. | <-- | PDCP PDU | - | - |
| 21 | CHECK: Does UE send the PDCP Data PDU#4 via RLC-AM RB with the following content: D/C field = 1 (PDCP Data PDU) , PDCP SN = 4, FU = 1, FR = 0, the value of Checksum is checked (Note 2)? Data is previously received data from PDU #4 after decompression. (Note 1) | --> | PDCP PDU | 2 | P |
| Note 1: The SS acknowledges the received data. Note 2: The SS compares the received value of Checksum and calculated value of Checksum based on the compression buffer. |  |  |  |  |  |

7.1.3.6.6.3.3	Specific message contents

Table 7.1.3.6.6.3.3-1: Paging (steps 7 and 16, Table 7.1.3.6.6.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-9 with condition NR_RRC_RESUME

Table 7.1.3.6.6.3.3-2: RRCResume (step 9, Table 7.1.3.6.6.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-17 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCResume ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcResume SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig |  |  |
| masterCellGroup |  | OCTET STRING (CONTAINING CellGroupConfig) | CellGroupConfig with condition RESUME |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.3.6.6.3.3-3: RadioBearerConfig (Table 7.1.3.6.6.3.3-2)

| Derivation Path: 38.508-1 [4], Table: 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| drb-Identity | Default DRB of the first PDU session |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.6.6.3.3-4: Void

Table 7.1.3.6.6.3.3-5: RRCResume (step 18, Table 7.1.3.6.6.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-17 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCResume ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcResume SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig |  |  |
| masterCellGroup |  | OCTET STRING (CONTAINING CellGroupConfig) | CellGroupConfig with condition RESUME |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.3.6.6.3.3-6: RadioBearerConfig (Table 7.1.3.6.6.3.3-5)

| Derivation Path: 38.508-1 [4], Table: 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| drb-Identity | Default DRB of the first PDU session |  |  |
| pdcp-Config | PDCP-Config |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.6.6.3.3-7: PDCP-Config (Table 7.1.3.6.6.3.3-6)

| Derivation Path: 38.508-1 [4], Table: 4.6.3-99 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-Config ::= SEQUENCE { |  |  |  |
| uplinkDataCompression-r17 CHOICE { |  |  |  |
| setup CHOICE { |  |  |  |
| drb-ContinueUDC | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.6.6.3.3-8: CellGroupConfig (Table 7.1.3.6.6.3.3-2, Table 7.1.3.6.6.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with condition PCell_change |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| spCellConfigCommon SEQUENCE { | Same as default ServingCellConfigCommon |  |  |
| physCellId | Physical cell Id of NR Cell 1 |  |  |
| } |  |  |  |
| rach-ConfigDedicated | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 7.1.3.6.7	PDCP UDC/ RRC reestablishment

7.1.3.6.7.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state with PDCP UDC configured and upon detecting radio link failure UE sends RRCReestablishmentRequest message }

ensure that {

when { UE receives the first RRCReconfiguration message after successful completion of the RRC re-establishment procedure and drb-ContinueUDC is not configured }

then { UE maintains its previous UDC configuration for the DRB and resets the UDC buffer and sets the FR field in UDC header of the first compressed PDU to 1 }

}

(2)

with { UE in NR RRC_CONNECTED state with PDCP UDC configured }

ensure that {

when { UE receives the first RRCReconfiguration message after successful completion of the RRC re-establishment procedure and drb-ContinueUDC is configured }

then { UE maintains its previous UDC configuration for the DRB and continues to use the previous UDC buffer }

}

7.1.3.6.7.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.323, clauses 5.1.2, TS 38.331, clauses 5.3.5.6.5, 5.3.7.2, 5.3.5.3. Unless otherwise stated these are Rel-17 requirements.

[TS 38.323, clause 5.1.2]

When upper layers request a PDCP entity re-establishment, the UE shall additionally perform once the procedures described in this clause for Uu or PC5 interface. After performing the procedures in this clause, the UE shall follow the procedures in clause 5.2.

When upper layers request a PDCP entity re-establishment, the transmitting PDCP entity shall:

-	for UM DRBs and AM DRBs, reset the ROHC protocol for uplink and start with an IR state in U-mode (as defined in RFC 3095 [8] and RFC 4815 [9]) if drb-ContinueROHC is not configured in TS 38.331 [3];

-	for UM DRBs and AM DRBs, reset the EHC protocol for uplink if drb-ContinueEHC-UL is not configured in TS 38.331 [3];

-	for AM DRBs, reset the UDC compression buffer to all zeros and prefill the dictionary if drb-ContinueUDC is not configured in TS 38.331 [3];

-	for SRBs and UM DRBs, set TX_NEXT to the initial value;

-	for SRBs, discard all stored PDCP SDUs and PDCP PDUs;

-	apply the ciphering algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	apply the integrity protection algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	for UM DRBs, for each PDCP SDU already associated with a PDCP SN but for which a corresponding PDU has not previously been submitted to lower layers, and;

-	for AM DRBs for Uu interface whose PDCP entities were suspended, from the first PDCP SDU for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by lower layers, for each PDCP SDU already associated with a PDCP SN:

-	consider the PDCP SDUs as received from upper layer;

-	perform transmission of the PDCP SDUs in ascending order of the COUNT value associated to the PDCP SDU prior to the PDCP re-establishment without restarting the discardTimer, as specified in clause 5.2.1;

-	for AM DRBs whose PDCP entities were not suspended, from the first PDCP SDU for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by lower layers, perform retransmission or transmission of all the PDCP SDUs already associated with PDCP SNs in ascending order of the COUNT values associated to the PDCP SDU prior to the PDCP entity re-establishment as specified below:

-	perform header compression of the PDCP SDU using ROHC as specified in the clause 5.7.4 and/or using EHC as specified in the clause 5.12.4;

-	If drb-ContinueUDC is configured and if the PDCP SDU has been compressed before:

-	submit the PDCP SDU previously compressed to integrity protection and ciphering function;

-	else:

-	perform uplink data compression of the PDCP SDU as specified in clause 5.14.4, and submit the PDCP SDU to integrity protection and ciphering function;

-	perform integrity protection and ciphering of the PDCP SDU using the COUNT value associated with this PDCP SDU as specified in the clause 5.9 and 5.8;

-	submit the resulting PDCP Data PDU to lower layer, as specified in clause 5.2.1.

[TS 38.331, clause 5.3.5.6.5]

…

1>	for each drb-Identity value included in the drb-ToAddModList that is part of the current UE configuration and not configured as DAPS bearer:

2>	if the reestablishPDCP is set:

3>	if target RAT of handover is E-UTRA/5GC; or

3>	if the UE is connected to E-UTRA/5GC:

4>	if the UE is capable of E-UTRA/5GC but not capable of NGEN-DC:

5>	if the PDCP entity of this DRB is not configured with cipheringDisabled:

6>	configure the PDCP entity with the ciphering algorithm and KUPenc key configured/derived as specified in TS 36.331 [10], clause 5.4.2.3, i.e. the ciphering configuration shall be applied to all subsequent PDCP PDUs received and sent by the UE;

4>	else (i.e., a UE capable of NGEN-DC):

5>	if the PDCP entity of this DRB is not configured with cipheringDisabled:

6>	configure the PDCP entity with the ciphering algorithm and KUPenc key associated with the master key (KeNB) or the secondary key (S-KgNB), as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent PDCP PDUs received and sent by the UE;

3>	else (i.e., UE connected to NR or UE connected to E-UTRA/EPC (in EN-DC or capable of EN-DC)):

4>	if the PDCP entity of this DRB is not configured with cipheringDisabled:

5>	configure the PDCP entity with the ciphering algorithm and KUPenc key associated with the master key (KeNB/ KgNB) or the secondary key (S-KgNB/S-KeNB), as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent PDCP PDUs received and sent by the UE;

4>	if the PDCP entity of this DRB is configured with integrityProtection:

5>	configure the PDCP entity with the integrity protection algorithms according to securityConfig and apply the KUPint key associated with the master key (KeNB/KgNB) or the secondary key (S-KgNB) as indicated in keyToUse;

3>	if drb-ContinueROHC is included in pdcp-Config:

4>	indicate to lower layer that drb-ContinueROHC is configured;

3>	if drb-ContinueEHC-DL is included in pdcp-Config:

4>	indicate to lower layer that drb-ContinueEHC-DL is configured;

3>	if drb-ContinueEHC-UL is included in pdcp-Config:

4>	indicate to lower layer that drb-ContinueEHC-UL is configured;

3>	if drb-ContinueUDC is included in pdcp-Config:

4>	indicate to lower layer that drb-ContinueUDC is configured;

3>	re-establish the PDCP entity of this DRB as specified in TS 38.323 [5], clause 5.1.2;

[TS 38.331, clause 5.3.7.2]

Upon initiation of the procedure, the UE shall:

1>	stop timer T310, if running;

1>	stop timer T312, if running;

1>	stop timer T304, if running;

1>	start timer T311;

1>	stop timer T316, if running;

1>	if UE is not configured with attemptCondReconfig:

2>	reset MAC;

2>	release spCellConfig, if configured;

2>	suspend all RBs, and BH RLC channels for IAB-MT, and Uu Relay RLC channels for L2 U2N Relay UE, except SRB0 and broadcast MRBs;

[TS 38.331, clause 5.3.5.3]

…

1>	else (RRCReconfiguration was received via SRB1):

2>	if the UE is in NR-DC and;

2>	if the RRCReconfiguration does not include the mrdc-SecondaryCellGroupConfig:

3>	if the RRCReconfiguration includes the scg-State:

4>	perform SCG deactivation as specified in 5.3.5.13b;

3>	else:

4>	perform SCG activation without SN message as specified in 5.3.5.13b1;

2>	if the reconfigurationWithSync was included in spCellConfig of an MCG:

3>	if ta-Report is configured with value enabled and the UE supports TA reporting:

4>	indicate TA report initiation to lower layers;

2>	submit the RRCReconfigurationComplete message via SRB1 to lower layers for transmission using the new configuration;

2>	if this is the first RRCReconfiguration message after successful completion of the RRC re-establishment procedure:

3>	resume SRB2, SRB4, DRBs, multicast MRB, and BH RLC channels for IAB-MT, that are suspended;

7.1.3.6.7.3	Test description

7.1.3.6.7.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.3.0 with UDC configuration except the following:

-	2 NR cells (NR Cell 1 and NR Cell 2) are configured with DRBs in RLC AM mode.

-	The cell power levels are configured as per the Table 7.1.3.6.7.3.1-1 (FR1) or Table 7.1.3.6.7.3.1-2 (FR2).

Table 7.1.3.6.7.3.1-1: Time instances of cell power level in FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | Off |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | Off | -88 |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -88 | Off |  |

Table 7.1.3.6.7.3.1-2: Time instances of cell power level in FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -82 | Off |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | Off | -82 |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -82 | Off |  |

7.1.3.6.7.3.2	Test procedure sequence

Table 7.1.3.6.7.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS creates 5 PDCP Data PDUs and the Next_PDCP_TX_SN is set to "0". | - | - | - | - |
| 2 | The SS sends the PDCP Data PDU#0 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 0. | <-- | PDCP PDU | - | - |
| 3 | UE sends the PDCP Data PDU#0 via RLC-AM RB on NR Cell 1 with the following content: D/C field = 1 (PDCP Data PDU) , PDCP SN = 0, FU = 1, FR = 0, Checksum = 1111. Data is previously received data from PDU #0 after decompression. (Note 1) | --> | PDCP PDU | - | - |
| 4 | The SS sends the PDCP Data PDU#1 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 1. | <-- | PDCP PDU | - | - |
| 5 | UE sends the PDCP Data PDU#1 via RLC-AM RB on NR Cell 1 with the following content: D/C field = 1 (PDCP Data PDU) , PDCP SN = 1, FU = 1, FR = 0, the value of Checksum is checked (Note 2). Data is previously received data from PDU #1 after decompression. (Note 1) | --> | PDCP PDU | - | - |
| 6 | The SS changes NR Cell 1and NR Cell 2 parameters according to the row "T1" in table 7.1.3.6.7.3.1-1/2 in order that the radio link quality of NR Cell 1 is degraded, and NR Cell 2 is suitable for camping. | - | - | - | - |
| 7 | UE sends RRCReestablishmentRequest message on NR Cell 2. | --> | NR RRC: RRCReestablishmentRequest | - | - |
| 8 | The SS transmits RRCReestablishment message. | <-- | NR RRC: RRCReestablishment | - | - |
| 9 | The UE transmits RRCReestablishmentComplete message. | --> | NR RRC: RRCReestablishmentComplete | - | - |
| 10 | The SS transmits an RRCReconfiguration message to resume existing radio bearer. | <-- | NR RRC: RRCReconfiguration | - | - |
| 11 | UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationtComplete | - | - |
| 12 | The SS sends the PDCP Data PDU#2 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 2. | <-- | PDCP PDU | - | - |
| 13 | CHECK: Does UE send the PDCP Data PDU#2 via RLC-AM RB on NR Cell 2 with the following content: D/C field = 1 (PDCP Data PDU) , PDCP SN = 2, FU = 1, FR = 1, Checksum = 1111? Data is previously received data from PDU #2 after decompression. (Note 1) | --> | PDCP PDU | 1 | P |
| 14 | The SS sends the PDCP Data PDU#3 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 3. | <-- | PDCP PDU | - | - |
| 15 | CHECK: Does UE send the PDCP Data PDU#4 via RLC-AM RB on NR Cell 2 with the following content: D/C field = 1 (PDCP Data PDU) , PDCP SN = 2, FU = 1, FR = 0, the value of Checksum is checked (Note 2)? Data is previously received data from PDU #3 after decompression. (Note 1) | --> | PDCP PDU | - | - |
| 16 | The SS changes NR Cell 1 and NR Cell 2 parameters according to the row "T2" in table 7.1.3.6.7.3.1-1/2 in order that the radio link quality of NR Cell 2 is degraded, and NR Cell 1 is suitable for camping. | - | - | - | - |
| 17 | UE sends RRCReestablishmentRequest message on NR Cell 1. | --> | NR RRC: RRCReestablishmentRequest | - | - |
| 18 | The SS transmits RRCReestablishment message. | <-- | NR RRC: RRCReestablishment | - | - |
| 19 | The UE transmits RRCReestablishmentComplete message. | --> | NR RRC: RRCReestablishmentComplete | - | - |
| 20 | The SS transmits an RRCReconfiguration message to resume existing radio bearer. | <-- | NR RRC: RRCReconfiguration | - | - |
| 21 | UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationtComplete | - | - |
| 22 | The SS sends the PDCP Data PDU#4 via RLC-AM RB with the following content to the UE: D/C field = 1 (PDCP Data PDU) and PDCP SN = 4. | <-- | PDCP PDU | - | - |
| 23 | CHECK: Does UE send the PDCP Data PDU#4 via RLC-AM RB on NR Cell 1 with the following content: D/C field = 1 (PDCP Data PDU) , PDCP SN = 2, FU = 1, FR = 0, the value of Checksum is checked (Note 2)? Data is previously received data from PDU #4 after decompression. (Note 1) | --> | PDCP PDU | 2 | P |
| Note 1: The SS acknowledges the received data. Note 2: The SS compares the received value of Checksum and calculated value of Checksum based on the compression buffer. |  |  |  |  |  |

7.1.3.6.7.3.3	Specific message contents

Table 7.1.3.6.7.3.3-1: RRCReestablishmentRequest (step 7, Table 7.1.3.6.7.3.2-1)

| Derivation Path: TS 38.508-1 [4] table 4.6.1-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReestablishmentRequest ::= SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of NR Cell 2 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS |  |  |
| } |  |  |  |
| reestablishmentCause | otherFailure |  |  |
| } |  |  |  |

Table 7.1.3.6.7.3.3-2: RRCReconfiguration (step 10, Table 7.1.3.6.7.3.2-1)

Derivation Path: TS 38.508-1[4], table 4.6.1-13 with condition REEST

Table 7.1.3.6.7.3.3-3: RRCReconfiguration (step 20, Table 7.1.3.6.7.3.2-1)

| Derivation Path: 38.508-1 [4], table 4.6.1-13 with condition REEST |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | OCTET STRING (CONTAINING CellGroupConfig) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.6.7.3.3-4: RadioBearerConfig (Table 7.1.3.6.7.3.3-3)

| Derivation Path: 38.508-1 [4], Table: 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| drb-Identity | Default DRB of the first PDU session |  |  |
| pdcp-Config | PDCP-Config |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.6.7.3.3-5: PDCP-Config (Table 7.1.3.6.7.3.3-4)

| Derivation Path: 38.508-1 [4], Table: 4.6.3-99 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-Config ::= SEQUENCE { |  |  |  |
| uplinkDataCompression-r17 CHOICE { |  |  |  |
| setup CHOICE { |  |  |  |
| drb-ContinueUDC | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.6.7.3.3-6: CellGroupConfig (Table 7.1.3.6.7.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with condition PCell_change |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| spCellConfigCommon SEQUENCE { | Same as default ServingCellConfigCommon |  |  |
| physCellId | Physical cell Id of NR Cell 1 |  |  |
| } |  |  |  |
| rach-ConfigDedicated | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 7.1.3.6.8	PDCP UDC/ PSCell addition / SCG DRB with UDC configuration/ NR-DC

7.1.3.6.8.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state with NR SCG DRB configured for UDC }

ensure that {

when { UE receives the PDCP SDUs on the DRB on AM RLC entity configured for SCG on PSCell  }

then { UE  transmits the PDCP SDUs on that DRB with UDC }

}

7.1.3.6.8.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.323, clauses 5.14.2, TS 38.331, clauses 5.3.5.3, 5.3.5.5.1, 5.3.5.5.7, 5.3.5.6.4 and 5.3.5.6.5. Unless otherwise stated these are Rel-17 requirements.

[TS 38.323, clause 5.14.2]

The PDCP entities associated with DRBs can be configured by upper layers, see TS 38.331 [3], to use UDC. If UDC is configured, the UE shall apply UDC compression function (details see Annex B) to process the received PDCP SDU from upper layers corresponding to the configured DRB. The size of compression buffer is configured by upper layers via bufferSize. If pre-defined dictionary is configured by upper layers, the UE shall first set the compression buffer to all zeros and then prefill the configured pre-defined dictionary in the compression buffer upon configuration of UDC. If pre-defined dictionary is not configured by upper layers, UE shall set the compression buffer to all zeros.

[TS 38.331, clause 5.3.5.3]

The UE shall perform the following actions upon reception of the RRCReconfiguration:

…

1>	if the RRCReconfiguration includes the secondaryCellGroup:

2>	perform the cell group configuration for the SCG according to 5.3.5.5;

1>	if the RRCReconfiguration includes the mrdc-SecondaryCellGroupConfig:

2>	if the mrdc-SecondaryCellGroupConfig is set to setup:

3>	if the mrdc-SecondaryCellGroupConfig includes mrdc-ReleaseAndAdd:

4>	perform MR-DC release as specified in clause 5.3.5.10;

3>	if the received mrdc-SecondaryCellGroup is set to nr-SCG:

4>	perform the RRC reconfiguration according to 5.3.5.3 for the RRCReconfiguration message included in nr-SCG;

3>	if the received mrdc-SecondaryCellGroup is set to eutra-SCG:

4>	perform the RRC connection reconfiguration as specified in TS 36.331 [10], clause 5.3.5.3 for the RRCConnectionReconfiguration message included in eutra-SCG;

[TS 38.331, clause 5.3.5.5.1]

…

1>	if the CellGroupConfig contains the spCellConfig:

2>	configure the SpCell as specified in 5.3.5.5.7;

[TS 38.331, clause 5.3.5.5.7]

The UE shall:

…

2>	if the SpCellConfig contains spCellConfigDedicated:

3>	configure the SpCell in accordance with the spCellConfigDedicated;

3>	consider the bandwidth part indicated in firstActiveUplinkBWP-Id, if included in the spCellConfigDedicated, to be the active uplink bandwidth part;

3>	if the firstActiveDownlinkBWP-Id is included in the spCellConfigDedicated:

4>	if the SpCellConfig is included in an RRCReconfiguration message contained in an NR or E-UTRA RRC message indicating that the SCG is deactivated:

5>	consider the bandwidth part indicated in firstActiveDownlinkBWP-Id to be the bandwidth part for Radio Link Monitoring, Beam Failure Detection and measurements;

4>	else:

5>	consider the bandwidth part indicated in firstActiveDownlinkBWP-Id to be the active downlink bandwidth part;

3>	if any of the reference signal(s) that are used for radio link monitoring are reconfigured by the received spCellConfigDedicated:

4>	stop timer T310 for the corresponding SpCell, if running;

4>	stop timer T312 for the corresponding SpCell, if running;

4>	reset the counters N310 and N311.

[TS 38.331, clause 5.3.5.6.5]

The UE shall:

1>	for each drb-Identity value included in the drb-ToAddModList that is not part of the current UE configuration (DRB establishment including the case when full configuration option is used):

2>	establish a PDCP entity and configure it in accordance with the received pdcp-Config;

7.1.3.6.8.3	Test description

7.1.3.6.8.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.3. for  NR-DC with UDC configuration of SCG DRB 0and:

-	NR Cell 1 is the PCell and NR Cell 10 is the PSCell.

-	System information combination NR-4 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

7.1.3.6.8.3.2	Test procedure sequence

Table 7.1.3.6.8.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-2 | Void | - |  | - | - |
| 3 | The SS sends a PDCP Data PDU#0 on the DRB on AM RLC entity configured for SCG on PSCell. | <-- | PDCP PDU | - | - |
| 4 | CHECK: Does the UE send the PDCP Data PDU#0 via RLC-AM RB on NR Cell 10 with the following content: D/C field = 1 (PDCP Data PDU) , PDCP SN = 0, FU = 1, FR = 0, Checksum = 1111? Data is previously received data from PDU #0 after decompression. (Note 1) | --> | PDCP PDU | 1 | P |
| Note 1: The SS acknowledges the received data. |  |  |  |  |  |

7.1.3.6.8.3.3	Specific message contents

Table 7.1.3.6.8.3.3-1: RRCReconfiguration (Preamble)

| Derivation Path: 38.508-1 [4], Table: 4.6.1-13 with condition NR-DC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| radioBearerConfig2 | OCTET STRING (CONTAINING RadioBearerConfig_SCG) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.6.8.3.3-2: RadioBearerConfig_SCG (Table 7.1.3.6.8.3.3-1)

| Derivation Path: 38.508-1 [4], Table: 4.6.3-132 with condition SCG_DRB and DRB2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| drb-Identity | DRB-Identity using condition DRB2 |  |  |
| pdcp-Config | PDCP-Config |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.6.8.3.3-3: PDCP-Config (Table 7.1.3.6.8.3.3-2)

Derivation Path: 38.508-1 [4], Table: 4.6.3-99 with condition UDC

##### 7.1.3.6.9	PDCP UDC/ PSCell addition / SCG DRB with UDC configuration/ NE-DC

7.1.3.6.9.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state with an E-UTRA SCG DRB configured for UDC }

ensure that {

when { UE receives anPDCP SDUs on the DRB on AM RLC entity configured for SCG on PSCell   }

then { UE transmits the PDCP SDUs on that DRB with UDC }

}

7.1.3.6.9.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.323, clauses 5.14.2, TS 38.331, clauses 5.3.5.3, 5.3.5.5.1, 5.3.5.5.7, 5.3.5.6.4, 5.3.5.6.5, TS 36.331, clauses 5.3.5.3, 5.3.10.3e, 5.3.10.10. Unless otherwise stated these are Rel-17 requirements.

[TS 38.323, clause 5.14.2]

The PDCP entities associated with DRBs can be configured by upper layers, see TS 38.331 [3], to use UDC. If UDC is configured, the UE shall apply UDC compression function (details see Annex B) to process the received PDCP SDU from upper layers corresponding to the configured DRB. The size of compression buffer is configured by upper layers via bufferSize. If pre-defined dictionary is configured by upper layers, the UE shall first set the compression buffer to all zeros and then prefill the configured pre-defined dictionary in the compression buffer upon configuration of UDC. If pre-defined dictionary is not configured by upper layers, UE shall set the compression buffer to all zeros.

[TS 38.331, clause 5.3.5.3]

The UE shall perform the following actions upon reception of the RRCReconfiguration:

…

1>	if the RRCReconfiguration includes the secondaryCellGroup:

2>	perform the cell group configuration for the SCG according to 5.3.5.5;

1>	if the RRCReconfiguration includes the mrdc-SecondaryCellGroupConfig:

2>	if the mrdc-SecondaryCellGroupConfig is set to setup:

3>	if the mrdc-SecondaryCellGroupConfig includes mrdc-ReleaseAndAdd:

4>	perform MR-DC release as specified in clause 5.3.5.10;

3>	if the received mrdc-SecondaryCellGroup is set to nr-SCG:

4>	perform the RRC reconfiguration according to 5.3.5.3 for the RRCReconfiguration message included in nr-SCG;

3>	if the received mrdc-SecondaryCellGroup is set to eutra-SCG:

4>	perform the RRC connection reconfiguration as specified in TS 36.331 [10], clause 5.3.5.3 for the RRCConnectionReconfiguration message included in eutra-SCG;

[TS 38.331, clause 5.3.5.5.1]

…

1>	if the CellGroupConfig contains the spCellConfig:

2>	configure the SpCell as specified in 5.3.5.5.7;

[TS 38.331, clause 5.3.5.5.7]

The UE shall:

…

2>	if the SpCellConfig contains spCellConfigDedicated:

3>	configure the SpCell in accordance with the spCellConfigDedicated;

3>	consider the bandwidth part indicated in firstActiveUplinkBWP-Id, if included in the spCellConfigDedicated, to be the active uplink bandwidth part;

3>	if the firstActiveDownlinkBWP-Id is included in the spCellConfigDedicated:

4>	if the SpCellConfig is included in an RRCReconfiguration message contained in an NR or E-UTRA RRC message indicating that the SCG is deactivated:

5>	consider the bandwidth part indicated in firstActiveDownlinkBWP-Id to be the bandwidth part for Radio Link Monitoring, Beam Failure Detection and measurements;

4>	else:

5>	consider the bandwidth part indicated in firstActiveDownlinkBWP-Id to be the active downlink bandwidth part;

3>	if any of the reference signal(s) that are used for radio link monitoring are reconfigured by the received spCellConfigDedicated:

4>	stop timer T310 for the corresponding SpCell, if running;

4>	stop timer T312 for the corresponding SpCell, if running;

4>	reset the counters N310 and N311.

[TS 38.331, clause 5.3.5.6.5]

The UE shall:

1>	for each drb-Identity value included in the drb-ToAddModList that is not part of the current UE configuration (DRB establishment including the case when full configuration option is used):

2>	establish a PDCP entity and configure it in accordance with the received pdcp-Config;

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

…

1>	if the received RRCConnectionReconfiguration includes the scg-Configuration; or

1>	if the current UE configuration includes one or more split DRBs configured with pdcp-Config and the received RRCConnectionReconfiguration includes radioResourceConfigDedicated including drb-ToAddModList:

2>	perform SCG reconfiguration as specified in 5.3.10.10;

[TS 36.331, clause 5.3.10.3e]

The UE shall:

1>	for each sCellGroupIndex value included in the sCellGroupToAddModList that is part of the current UE configuration (SCell group modification):

2>	for each sCellIndex value included in the sCellToReleaseList that is part of the SCell group indicated by sCellGroupIndex (SCell deletion from SCell group):

3>	consider the sCellConfigCommon of the SCell group to be not applicable for the SCell;

3>	consider the SCell not to be part of the SCell group indicated by sCellGroupIndex

2>	for each sCellIndex value included in the sCellToAddModList that is not part of the SCell group indicated by sCellGroupIndex (SCell addition to SCell group):

3>	consider the SCell to be part of the SCell group indicated by sCellGroupIndex;

3>	apply the SCell configuration for parameters not already configured as part of the current SCell configuration in accordance with the sCellConfigCommon for the SCell group;

2>	if sCellConfigCommon is included (modify the SCell group configuration):

3> for each SCell that is part of the current SCell group indicated by sCellGroupIndex:

4> apply the SCell configuration for parameters not already configured as part of the current SCell configuration in accordance with the sCellConfigCommon for the SCell group;

1>	for each sCellGroupIndex value included in the sCellGroupToAddModList that is not part of the current UE configuration (SCell group addition):

2>	for each sCellIndex value included in the sCellToAddModList (SCell addition to the group):

3>	consider the SCell to be part of the SCell group indicated by sCellGroupIndex

3> apply the SCell configuration for parameters not already configured as part of the current SCell configuration in accordance with the sCellConfigCommon for the SCell group;

[TS 36.331, clause 5.3.10.10]

The UE shall:

…

1>	else:

2>	if scg-ConfigPartMCG is received and includes the scg-Counter:

3>	update the S-KeNB key based on the KeNB key and using the received scg-Counter value, as specified in TS 33.401 [32];

3>	derive the KUPenc key associated with the cipheringAlgorithmSCG included in mobilityControlInfoSCG within the received scg-ConfigPartSCG, as specified in TS 33.401 [32];

3>	configure lower layers to apply the ciphering algorithm and the KUPenc key;

2>	if scg-ConfigPartSCG is received and includes the radioResourceConfigDedicatedSCG:

3>	reconfigure the dedicated radio resource configuration for the SCG as specified in 5.3.10.11;

2>	if the current UE configuration includes one or more split or SCG DRBs and the received RRCConnectionReconfiguration message includes radioResourceConfigDedicated including drb-ToAddModList:

3>	reconfigure the SCG or split DRB by drb-ToAddModList as specified in 5.3.10.12;

2>	if scg-ConfigPartSCG is received and includes measConfigSN:

3>	for measConfigSN perform the actions as specified in 5.5.2 for measConfig unless explicitly stated otherwise;

2>	if scg-ConfigPartSCG is received and includes the sCellToReleaseListSCG:

3>	perform SCell release for the SCG as specified in 5.3.10.3a;

2>	if scg-ConfigPartSCG is received and includes the pSCellToAddMod:

3>	perform PSCell addition or modification as specified in 5.3.10.3c;

NOTE 0:	This procedure is also used to release the PSCell e.g. PSCell change, SI change for the PSCell.

2>	if scg-ConfigPartSCG is received and includes the sCellToAddModListSCG:

3>	perform SCell addition or modification as specified in 5.3.10.3b;

2>	configure lower layers in accordance with mobilityControlInfoSCG, if received;

2>	if rach-SkipSCG is configured:

3>	configure lower layers to apply the rach-SkipSCG for the target SCG, as specified in TS 36.213 [23] and TS 36.321 [6];

2>	if scg-ConfigPartSCG is received and includes the mobilityControlInfoSCG (i.e. SCG change):

3>	resume all SCG DRBs and resume SCG transmission for split DRBs, if suspended;

3>	stop timer T313, if running;

3>	start timer T307 with the timer value set to t307, as included in the mobilityControlInfoSCG, if makeBeforeBreakSCG is not configured;

3>	start synchronising to the DL of the target PSCell;

3>	initiate the random access procedure on the PSCell, as specified in TS 36.321 [6], if rach-SkipSCG is not configured:

NOTE 1:	The UE is not required to determine the SFN of the target PSCell by acquiring system information from that cell before performing RACH access in the target PSCell.

3>	the procedure ends, except that the following actions are performed when MAC successfully completes the random access procedure on the PSCell or when MAC indicates the successful reception of a PDCCH transmission addressed to C-RNTI and if rach-skipSCG is configured:

4>	stop timer T307;

4>	release rach-SkipSCG;

4>	apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the target PSCell, if any;

4>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target PSCell (e.g. periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target PSCell;

NOTE 2:	Whenever the UE shall setup or reconfigure a configuration in accordance with a field that is received it applies the new configuration, except for the cases addressed by the above statements.

7.1.3.6.9.3	Test description

7.1.3.6.9.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.3.0 for NE-DC with UDC configuration of SCG DRB.

7.1.3.6.9.3.2	Test procedure sequence

Table 7.1.3.6.9.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-2 | Void | - |  | - | - |
| 3 | The SS sends a PDCP Data PDU#0 on the DRB on AM RLC entity configured for SCG on PSCell. | <-- | PDCP PDU | - | - |
| 4 | CHECK: Does UE send the PDCP Data PDU#0 via RLC-AM RB on E-UTRA Cell 1 with the following content: D/C field = 1 (PDCP Data PDU) , PDCP SN = 0, FU = 1, FR = 0, Checksum = 1111? Data is previously received data from PDU #0 after decompression. (Note 1) | --> | PDCP PDU | 1 | P |
| Note 1: The SS acknowledges the received data. |  |  |  |  |  |

7.1.3.6.9.3.3	Specific message contents

Table 7.1.3.6.9.3.3-1: RRCReconfiguration (Preamble)

Derivation Path: 38.508-1 [4], Table: 4.6.1-13 with condition NE-DC

Table 7.1.3.6.9.3.3-2: RRCConnectionReconfiguration (Table 7.1.3.6.9.3.3-1)

| Derivation Path: 36.508 [7], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionReconfiguration-v1250-IEs |  |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionReconfiguration-v1310-IEs |  |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionReconfiguration-v1430-IEs |  |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionReconfiguration-v1510-IEs |  |
| nr-RadioBearerConfig1-r15 | OCTET STRING (CONTAINING RadioBearerConfig_SCG) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.6.9.3.3-3: RadioBearerConfig_SCG (Table 7.1.3.6.9.3.3-2)

| Derivation Path: 38.508-1 [4], Table: 4.6.3-132 with condition SCG_DRB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| drb-Identity | DRB ID used for NE-DC SCG |  |  |
| pdcp-Config | PDCP-Config |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.3.6.9.3.3-4: PDCP-Config (Table 7.1.3.6.9.3.3-3)

Derivation Path: 38.508-1 [4], Table: 4.6.3-99 with condition UDC

### 7.1.4	SDAP

#### 7.1.4.1	SDAP Data Transfer and PDU Header Handling UL/DL

7.1.4.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with multiple DRB's established, each mapping more than one QoS flow }

ensure that {

when { UE receives an SDAP PDU with SDAP header }

then { UE SDAP entity retrieves the SDAP SDU from the SDAP PDU and delivers it to upper layer }

}

(2)

with { UE in RRC_CONNECTED state with multiple DRB's established configured with UL SDAP header, each mapping more than one QoS flow configured by RRC }

ensure that {

when { UE has to transmit a SDAP PDU with header to be included }

then {UE builds an SDAP PDU from the SDAP SDU including the header, and maps it to the DRB as per stored DRB mapping rule for the QoS flow }

}

(3)

with { UE in RRC_CONNECTED state with multiple DRB's and QoS flows established }

ensure that {

when { UE receives a SDAP PDU with SDAP header and RDI fields set to 1 }

then { UE stores the QoS flow to DRB mapping of the DL SDAP PDU as the QoS flow to DRB mapping rule for the UL and uses it for further UL SDAP PDU transmissions }

}

(4)

with { UE in RRC_CONNECTED state with multiple DRB's and QoS flows established }

ensure that {

when { UE receives a SDAP PDU with SDAP header and RDI field set to 1 and the stored QoS flow to DRB mapping rule for the QoS flow is different from the QoS flow to DRB mapping of the DL SDAP data PDU }

then { UE stores the QoS flow to DRB mapping of the DL SDAP PDU as the QoS flow to DRB mapping rule for the UL, to be used for further UL SDAP PDU transmissions and transmits an end-marker control PDU for the QoS flow on the old DRB }

}

(5)

with { UE in RRC_CONNECTED state with multiple DRB's and QoS flows established with QoS flow to DRB mapping }

ensure that {

when { UE receives a message which configures a new QoS flow to DRB mapping, different from the existing mapping }

then { UE stores the QoS flow to DRB mapping to be used for further UL SDAP PDU transmissions and transmits an end-marker control PDU for the QoS flow on the old DRB }

}

7.1.4.1.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 37.324, clauses 5.2.1, 5.2.2, 5.3.1, 5.3.2, 6.2.2.1, 6.2.2.2, 6.2.2.3, 6.2.3 and 6.3.4, TS 24.501 clause 6.2.5.1.3. Unless otherwise stated these are Rel-15 requirements.

[TS 37.324 clause 5.2.1]

At the reception of an SDAP SDU from upper layer for a QoS flow, the transmitting SDAP entity shall:

-	if there is no stored QoS flow to DRB mapping rule for the QoS flow as specified in the subclause 5.3:

-	map the SDAP SDU to the default DRB;

-	else:

-	map the SDAP SDU to the DRB according to the stored QoS flow to DRB mapping rule;

-	if the DRB to which the SDAP SDU is mapped is configured by RRC (3GPP TS 38.331 [3]) with the presence of SDAP header,

-	construct the UL SDAP data PDU as specified in the subclause 6.2.2.3;

-	else:

-	construct the UL SDAP data PDU as specified in the subclause 6.2.2.1;

-	submit the constructed UL SDAP data PDU to the lower layers.

NOTE 1:	UE behaviour is not defined if there is neither a default DRB nor a stored QoS flow to DRB mapping rule for the QoS flow.

NOTE 2:	Default DRB is always configured with UL SDAP header (3GPP TS 38.331 [3]).

[TS 37.324 clause 5.2.2]

At the reception of an SDAP data PDU from lower layers for a QoS flow, the receiving SDAP entity shall:

-	if the DRB from which this SDAP data PDU is received is configured by RRC (3GPP TS 38.331 [3]) with the presence of SDAP header:

-	perform reflective QoS flow to DRB mapping as specified in the subclause 5.3.2;

-	perform RQI handling as specified in the subclause 5.4;

-	retrieve the SDAP SDU from the DL SDAP data PDU as specified in the subclause 6.2.2.2.

-	else:

-	retrieve the SDAP SDU from the DL SDAP data PDU as specified in the subclause 6.2.2.1;

-	deliver the retrieved SDAP SDU to the upper layer.

[TS 37.324 clause 5.3.1]

When RRC (3GPP TS 38.331 [3]) configures an UL QoS flow to DRB mapping rule for a QoS flow, the SDAP entity shall:

-	if the SDAP entity has already been established and there is no stored QoS flow to DRB mapping rule for the QoS flow and a default DRB is configured:

-	construct an end-marker control PDU, as specified in the subclause 6.2.3, for the QoS flow;

-	map the end-marker control PDU to the default DRB;

-	submit the end-marker control PDU to the lower layers.

-	if the stored UL QoS flow to DRB mapping rule is different from the configured QoS flow to DRB mapping rule for the QoS flow and the DRB according to the stored QoS flow to DRB mapping rule is configured by RRC (3GPP TS 38.331 [3]) with the presence of UL SDAP header:

-	construct an end-marker control PDU, as specified in the subclause 6.2.3, for the QoS flow;

-	map the end-marker control PDU to the DRB according to the stored QoS flow to DRB mapping rule;

-	submit the end-marker control PDU to the lower layers.

-	store the configured UL QoS flow to DRB mapping rule for the QoS flow.

When RRC (3GPP TS 38.331 [3]) releases an UL QoS flow to DRB mapping rule for a QoS flow, the SDAP entity shall:

-	remove the UL QoS flow to DRB mapping rule for the QoS flow.

[TS 37.324 clause 5.3.2]

For each received DL SDAP  dataPDU with RDI set to 1, the SDAP entity shall:

-	process the QFI field in the SDAP header and determine the QoS flow;

-	if there is no stored QoS flow to DRB mapping rule for the QoS flow and a default DRB is configured:

-	construct an end-marker control PDU, as specified in the subclause 6.2.3, for the QoS flow;

-	map the end-marker control PDU to the default DRB;

-	submit the end-marker control PDU to the lower layers;

-	if the stored QoS flow to DRB mapping rule for the QoS flow is different from the QoS flow to DRB mapping of the DL SDAP data PDU and the DRB according to the stored QoS flow to DRB mapping rule is configured by RRC (3GPP TS 38.331 [3]) with the presence of UL SDAP header:

-	construct an end-marker control PDU, as specified in the subclause 6.2.3, for the QoS flow;

-	map the end-marker control PDU to the DRB according to the stored QoS flow to DRB mapping rule;

-	submit the end-marker control PDU to the lower layers;

-	store the QoS flow to DRB mapping of the DL SDAP data PDU as the QoS flow to DRB mapping rule for the UL.

[TS 37.324 clause 6.2.2.1]

An SDAP PDU consists only of a data field and does not consist of any SDAP header, as described in Figure 6.2.2.1-1.

Figure 6.2.2.1-1: SDAP Data PDU format without SDAP header

[TS 37.324 clause 6.2.2.2]

Figure 6.2.2.2 – 1 shows the format of SDAP Data PDU of DL with SDAP header being configured.

Figure 6.2.2.2-1: DL SDAP Data PDU format with SDAP header

[TS 37.324 clause 6.2.2.3]

Figure 6.2.2.3 – 1 shows the format of SDAP Data PDU of UL with SDAP header being configured.

Figure 6.2.2.3-1: UL SDAP Data PDU format with SDAP header

[TS 37.324 clause 6.2.3]

Figure 6.2.3 – 1 shows the format of End-Marker Control PDU.

Figure 6.2.2.3-1: UL SDAP Data PDU format with SDAP header

[TS 37.324 clause 6.3.4]

Length: 6 bits

The QFI field indicates the ID of the QoS flow (3GPP TS 23.501 [4]) to which the SDAP PDU belongs.

[TS 24.501 clause 6.2.5.1.3]

For PDU session of IPv4, IPv6, IPv4v6 or Ethernet PDU session type, upon receiving an UL user data packet from the upper layers for transmission via a PDU session, the UE shall attempt to associate the UL user data packet with:

a)	the QFI of a signalled QoS rule associated with the PDU session which has a set of packet filters containing a packet filter for UL direction matching the UL user data packet or containing a packet filter for both UL and DL directions matching the UL user data packet; or

b)	the QFI of a derived QoS rule associated with the PDU session which has the packet filter for UL direction matching the UL user data packet;

by evaluating the QoS rules in increasing order of their precedence values until the UL user data packet is associated with a QFI or all QoS rules are evaluated.

For PDU session of unstructured PDU session type, upon receiving an UL user data packet from the upper layers for transmission via a PDU session, the UE shall associate the UL user data packet with the QFI of the default QoS rule associated with the PDU session.

If the UL user data packet is associated with a QFI, the UE shall pass the QFI along the UL user data packet to the lower layers for transmission.

NOTE:	Marking of the UL user data packet with the QFI is performed by the lower layers.

If all QoS rules are evaluated and the UL user data packet is not associated with a QFI, the UE shall discard the UL user data packet.

7.1.4.1.3	Test description

7.1.4.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

UE:

-	None.

Preamble:

-	The UE is in 5GS state 3N-A with one PDU session active according to TS 38.508-1 [4], clause 4.4A.3, Table 4.4A.3-1 and using the message condition UE TEST LOOP MODE B active with IP PDU delay = 1 second, to return one SDAP SDU per DL SDAP SDU. 2 DRBs are configured where DRB j is defined as default DRB. The NAS QoS rules for the QoS flows with QFI=1, QFI=2, QFI=5 and QFI=6 are configured. QoS flows with QFI=5 and QFI=6 are mapped to DRB j, QoS flows with QFI=1 and QFI=2 are mapped to DRB k.

7.1.4.1.3.2	Test procedure sequence

Table 7.1.4.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS sends the SDAP Data PDU with SDAP header on DRB k and the following content to the UE: RDI=0. RQI=0, QFI=2. | <-- | SDAP DL Data PDU | - | - |
| 2 | Check: Does the UE re-transmit SDAP Data PDU on DRB k with SDAP header as per the stored DRB mapping flow with QFI=2? | --> | SDAP UL Data PDU | 1,2 | P |
| 2A | The SS transmits an OPEN UE TEST LOOP message. | <-- | OPEN UE TEST LOOP | - | - |
| 2B | The UE transmits an OPEN UE TEST LOOP COMPLETE message. | --> | OPEN UE TEST LOOP COMPLETE | - | - |
| 2C | The SS transmits a CLOSE UE TEST LOOP Message with IP PDU delay = 1 second. | <-- | CLOSE UE TEST LOOP | - | - |
| 2D | The UE transmits a CLOSE UE TEST LOOP COMPLETE message. | --> | CLOSE UE TEST LOOP COMPLETE | - | - |
| 3 | The SS sends the SDAP Data PDU with SDAP header on DRB k and the following content to the UE: RDI=1, RQI=0, QFI=5. | <-- | SDAP DL Data PDU | - | - |
| - | EXCEPTION: In parallel to the event described in step 4 the events specified in Table 7.1.4.1.3.2-2 shall take place. | - | - | - | - |
| 4 | Check: Does the UE re-transmit SDAP Data PDU on DRB k with SDAP header as per the stored DRB mapping Flow with QFI=5? | --> | SDAP UL Data PDU | 3 | P |
| 4A | The SS transmits an OPEN UE TEST LOOP message. | <-- | OPEN UE TEST LOOP | - | - |
| 4B | The UE transmits an OPEN UE TEST LOOP COMPLETE message. | --> | OPEN UE TEST LOOP COMPLETE | - | - |
| 4C | The SS transmits a CLOSE UE TEST LOOP Message with IP PDU delay = 1 second. | <-- | CLOSE UE TEST LOOP | - | - |
| 4D | The UE transmits a CLOSE UE TEST LOOP COMPLETE message. | --> | CLOSE UE TEST LOOP COMPLETE | - | - |
| 5 | The SS transmits an RRCReconfiguration message including a PDU SESSION MODIFICATION COMMAND | <-- | RRCReconfiguration (PDU SESSION MODIFICATION COMMAND) | - | - |
| - | EXCEPTION: In parallel to the event described in step 6 the events specified in Tables 7.1.4.1.3.2-3 and 7.1.4.1.3.2-4 shall take place. | - | - | - | - |
| 6 | The UE transmits an RRCReconfigurationComplete message. | --> | RRCReconfigurationComplete | - | - |
| 7 | The SS sends the SDAP Data PDU with SDAP header on DRB k and the following content to the UE: RDI=0. RQI=0, QFI=4. | <-- | SDAP DL Data PDU | - | - |
| 8 | Check: Does the UE re-transmit SDAP Data PDU on DRB k with SDAP header as per the stored DRB mapping Flow with QFI=4? | --> | SDAP UL Data PDU | 5 | P |

Table 7.1.4.1.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE transmit End-Marker Control PDU on DRB j for QFI=5? | --> | SDAP UL Control PDU | 4 | P |

Table 7.1.4.1.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE transmit End-Marker Control PDU on DRB j for QFI=4? | --> | SDAP UL Control PDU | 5 | P |

Table 7.1.4.1.3.2-4: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE Transmits PDU SESSION MODIFICATION COMPLETE | --> | ULInformationTransfer (PDU SESSION MODIFICATION COMPLETE) | - | - |

7.1.4.1.3.3	Specific message contents

Table 7.1.4.1.3.3-1: RadioBearerConfig-DRB (Preamble)

| Derivation Path: TS 38.508-1 [4], table 4.6.3-132 and condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 2 entries |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config SEQUENCE { |  |  |  |
| pdu-Session | The same as the PDU session ID in PDU SESSION ESTABLISHMENT REQUEST |  |  |
| sdap-HeaderDL | present |  |  |
| sdap-HeaderUL | present |  |  |
| defaultDRB | false |  |  |
| mappedQoS-FlowsToAdd SEQUENCE { |  |  |  |
| QFI | 1 |  |  |
| QFI | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| drb-Identity | k | k is allocated according to internal TTCN mapping |  |
| } |  |  |  |
| DRB-ToAddMod[2] SEQUENCE { |  | entry 2 |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config SEQUENCE { |  |  |  |
| pdu-Session | The same as the PDU session ID in PDU SESSION ESTABLISHMENT REQUEST |  |  |
| sdap-HeaderDL | present |  |  |
| sdap-HeaderUL | present |  |  |
| defaultDRB | true |  |  |
| mappedQoS-FlowsToAdd SEQUENCE { |  |  |  |
| QFI | 5 |  |  |
| QFI | 6 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| drb-Identity | j | j is allocated according to internal TTCN mapping |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.4.1.3.3-2: RadioBearerConfig-DRB (step 5, Table 7.1.4.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], table 4.6.3-132 and condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { | - | - |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry | BID is the total number of established DRBs in the UE, before applying the contents of this IE |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config SEQUENCE { |  |  |  |
| pdu-Session | The same as the PDU session ID in PDU SESSION ESTABLISHMENT REQUEST |  |  |
| sdap-HeaderDL | present |  |  |
| sdap-HeaderUL | present |  |  |
| defaultDRB | false |  |  |
| mappedQoS-FlowsToAdd SEQUENCE { |  |  |  |
| QFI | 4 |  |  |
| } |  |  |  |
| } |  |  |  |
| drb-Identity | k | k is allocated according to internal TTCN mapping |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.4.1.3.3-3: PDU SESSION MODIFICATION COMMAND (step 5, Table 7.1.4.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], table 4.7.2-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDU session ID | The same as the PDU session ID in PDU SESSION ESTABLISHMENT REQUEST |  |  |
| Authorized QoS rules | 1 entry |  |  |
| QoS rule [1] | Reference QoS rule #4a as defined in TS 38.508-1 [4], Table 4.8.2.1-4a. | QFI=4 |  |
| Authorized QoS flow descriptions | 1 entry |  |  |
| QoS flow [1] | Reference QoS flow #2a as defined in TS 38.508-1 [4], Table 4.8.2.3-2a. | QFI=4 |  |

Table 7.1.4.1.3.3-4: PDU SESSION ESTABLISHMENT ACCEPT (Preamble)

| Derivation Path: TS 38.508-1 [4], table 4.7.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDU session ID | The same as the PDU session ID in PDU SESSION ESTABLISHMENT REQUEST |  |  |
| Authorized QoS rules | 4 entries |  |  |
| QoS rule [1] | Reference QoS rule #1 as defined in TS 38.508-1 [4], Table 4.8.2.1-1 | QFI=1 |  |
| QoS rule [2] | Reference QoS rule #4 as defined in Table 7.1.4.1.3.3-7 | QFI=2 |  |
| QoS rule [3] | Reference QoS rule #5 as defined in TS 38.508-1 [4], Table 4.8.2.1-5 | QFI=5 |  |
| QoS rule [4] | Reference QoS rule #6 as defined in TS 38.508-1 [4], Table 4.8.2.1-6 | QFI=6 |  |
| Mapped EPS Bearer contexts | Not Present |  |  |
| Authorized QoS flow descriptions | 4 entries |  |  |
| QoS flow [1] | Reference QoS flow #1 as defined in Table 7.1.4.1.3.3-5 | QFI=1 |  |
| QoS flow [2] | Reference QoS flow #2 as defined in Table 7.1.4.1.3.3-6 | QFI=2 |  |
| QoS flow [3] | Reference QoS flow #3 as defined in TS 38.508-1 [4], Table 4.8.2.3-3 | QFI=5 |  |
| QoS flow [4] | Reference QoS flow #4 as defined in TS 38.508-1 [4], Table 4.8.2.3-4 | QFI=6 |  |

Table 7.1.4.1.3.3-5: Reference QoS flow #1 (Preamble)

| Derivation Path: TS 38.508-1 [4], table 4.8.2.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description | 1 entry |  |  |
| QFI | ‘00 0001’B | QFI 1 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| 5QI | ‘0000 1001’B | 5QI 9 |  |

Table 7.1.4.1.3.3-6: Reference QoS flow #2 (Preamble)

| Derivation Path: TS 38.508-1 [4], table 4.8.2.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description | 1 entry |  |  |
| QFI | ‘00 0010’B | QFI 2 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| 5QI | ‘0000 0101’B | 5QI 5 |  |

Table 7.1.4.1.3.3-7: Reference QoS rule #4

| Derivation Path: TS 38.508-1[4], table 4.8.2.1-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS rules |  |  |  |
| QoS rule |  |  |  |
| DQR bit | ‘0’B |  |  |

#### 7.1.4.2	SDAP Data Transfer handling without Header UL/DL

7.1.4.2.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with multiple DRB's established. SDAP configured without header and no stored QoS flow mapping }

ensure that {

when { UE receives a SDAP SDU from upper layers }

then { UE SDAP entity transmits the SDAP PDU with header on default DRB }

}

(2)

with { UE in RRC_CONNECTED state with multiple DRB's established. SDAP configured without header and no stored QoS flow mapping }

ensure that {

when { UE SDAP recieves from RRC new Qos Flow mapping }

then { UE SDAP entity transmits an end-marker control PDU for the QoS flow on default DRB }

}

(3)

with { UE in RRC_CONNECTED state with multiple DRB's established. SDAP configured without header and stored QoS flow mapping configured by RRC }

ensure that {

when { UE receives a SDAP SDU from upper layers }

then { UE SDAP entity transmits the SDAP PDU without header on non default DRB as per configured QoS flow mapping }

}

7.1.4.2.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 37.324, clauses 5.2.1, 5.2.2, 5.3.1, 6.2.2.1 and 6.2.3, TS 24.501 clause 6.2.5.1.3. Unless otherwise stated these are Rel-15 requirements.

[TS 37.324 clause 5.2.1]

At the reception of an SDAP SDU from upper layer for a QoS flow, the transmitting SDAP entity shall:

-	if there is no stored QoS flow to DRB mapping rule for the QoS flow as specified in the subclause 5.3:

-	map the SDAP SDU to the default DRB;

-	else:

-	map the SDAP SDU to the DRB according to the stored QoS flow to DRB mapping rule;

-	if the DRB to which the SDAP SDU is mapped is configured by RRC (3GPP TS 38.331 [3]) with the presence of SDAP header,

-	construct the UL SDAP data PDU as specified in the subclause 6.2.2.3;

-	else:

-	construct the UL SDAP data PDU as specified in the subclause 6.2.2.1;

-	submit the constructed UL SDAP data PDU to the lower layers.

NOTE 1:	UE behaviour is not defined if there is neither a default DRB nor a stored QoS flow to DRB mapping rule for the QoS flow.

NOTE 2:	Default DRB is always configured with UL SDAP header (3GPP TS 38.331 [3]).

[TS 37.324 clause 5.2.2]

At the reception of an SDAP data PDU from lower layers for a QoS flow, the receiving SDAP entity shall:

-	if the DRB from which this SDAP data PDU is received is configured by RRC (3GPP TS 38.331 [3]) with the presence of SDAP header:

-	perform reflective QoS flow to DRB mapping as specified in the subclause 5.3.2;

-	perform RQI handling as specified in the subclause 5.4;

-	retrieve the SDAP SDU from the DL SDAP data PDU as specified in the subclause 6.2.2.2.

-	else:

-	retrieve the SDAP SDU from the DL SDAP data PDU as specified in the subclause 6.2.2.1;

-	deliver the retrieved SDAP SDU to the upper layer.

[TS 37.324 clause 5.3.1]

When RRC (3GPP TS 38.331 [3]) configures an UL QoS flow to DRB mapping rule for a QoS flow, the SDAP entity shall:

-	if the SDAP entity has already been established and there is no stored QoS flow to DRB mapping rule for the QoS flow and a default DRB is configured:

-	construct an end-marker control PDU, as specified in the subclause 6.2.3, for the QoS flow;

-	map the end-marker control PDU to the default DRB;

-	submit the end-marker control PDU to the lower layers.

-	if the stored UL QoS flow to DRB mapping rule is different from the configured QoS flow to DRB mapping rule for the QoS flow and the DRB according to the stored QoS flow to DRB mapping rule is configured by RRC (3GPP TS 38.331 [3]) with the presence of UL SDAP header:

-	construct an end-marker control PDU, as specified in the subclause 6.2.3, for the QoS flow;

-	map the end-marker control PDU to the DRB according to the stored QoS flow to DRB mapping rule;

-	submit the end-marker control PDU to the lower layers.

-	store the configured UL QoS flow to DRB mapping rule for the QoS flow.

When RRC (3GPP TS 38.331 [3]) releases an UL QoS flow to DRB mapping rule for a QoS flow, the SDAP entity shall:

-	remove the UL QoS flow to DRB mapping rule for the QoS flow.

[TS 37.324 clause 6.2.2.1]

An SDAP PDU consists only of a data field and does not consist of any SDAP header, as described in Figure 6.2.2.1-1.

Figure 6.2.2.1-1: SDAP Data PDU format without SDAP header

[TS 37.324 clause 6.2.3]

Figure 6.2.3 – 1 shows the format of End-Marker Control PDU.

Figure 6.2.3-1: End-Marker Control PDU

[TS 24.501 clause 6.2.5.1.3]

For PDU session of IPv4, IPv6, IPv4v6 or Ethernet PDU session type, upon receiving an UL user data packet from the upper layers for transmission via a PDU session, the UE shall attempt to associate the UL user data packet with:

a)	the QFI of a signalled QoS rule associated with the PDU session which has a set of packet filters containing a packet filter for UL direction matching the UL user data packet or containing a packet filter for both UL and DL directions matching the UL user data packet; or

b)	the QFI of a derived QoS rule associated with the PDU session which has the packet filter for UL direction matching the UL user data packet;

by evaluating the QoS rules in increasing order of their precedence values until the UL user data packet is associated with a QFI or all QoS rules are evaluated.

For PDU session of unstructured PDU session type, upon receiving an UL user data packet from the upper layers for transmission via a PDU session, the UE shall associate the UL user data packet with the QFI of the default QoS rule associated with the PDU session.

If the UL user data packet is associated with a QFI, the UE shall pass the QFI along the UL user data packet to the lower layers for transmission.

NOTE:	Marking of the UL user data packet with the QFI is performed by the lower layers.

If all QoS rules are evaluated and the UL user data packet is not associated with a QFI, the UE shall discard the UL user data packet.

7.1.4.2.3	Test description

7.1.4.2.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

UE:

-	None.

Preamble:

-	The UE is in 5GS state 3N-A with one PDU session active according to TS 38.508-1 [4], clause 4.4A.3, Table 4.4A.3-1 and using the message condition UE TEST LOOP MODE B active to return one UL SDAP SDU per DL SDAP SDU. 2 DRBs are configured where DRBj is defined as default DRB. The NAS QoS rules for QoS flows QFI = 5 and QFI = 2 are configured. The 'mappedQoS-Flows' is empty for both DRB's for SDAP layer.

7.1.4.2.3.2	Test procedure sequence

Table 7.1.4.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS sends the SDAP Data PDU without SDAP header on DRB k for QFI =2. | <-- | SDAP DL Data PDU | - | - |
| 2 | Check: Does the UE transmit SDAP Data PDU on DRB j, which is default DRB, with SDAP header including QFI=2? | --> | SDAP UL Data PDU | 1 | P |
| 3 | The SS sends the SDAP Data PDU without SDAP header on DRB j for QFI 5. | <-- | SDAP DL Data PDU | - | - |
| 4 | Check: Does the UE transmit SDAP Data PDU on DRB j, which is default DRB, with SDAP header including QFI=5? | --> | SDAP UL Data PDU | 1 | P |
| 5 | The SS transmits an NR RRCReconfiguration message to configure QoS Flow rules | <-- | (RRCReconfiguration) | - | - |
| - | EXCEPTION: In parallel to the event described in step 6 the events specified in Table 7.1.4.2.3.2-2 shall take place. | - | - | - | - |
| 6 | The UE transmit an NR RRCReconfigurationComplete message. | --> | (RRCReconfigurationComplete) | - | - |
| 7 | The SS sends the SDAP Data PDU without SDAP header on DRB k for QFI =2. | <-- | SDAP DL Data PDU | - | - |
| 8 | Check: Does the UE transmit SDAP Data PDU on DRB k? | --> | SDAP UL Data PDU | 3 | P |
| 9 | The SS sends the SDAP Data PDU without SDAP header on DRB j for QFI 5. | <-- | SDAP DL Data PDU | - | - |
| 10 | Check: Does the UE transmit SDAP Data PDU on DRB j, with SDAP header including QFI=5? | --> | SDAP UL Data PDU | 3 | P |

Table 7.1.4.2.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE transmit End-Marker Control PDU on DRBj for QFI=2? | --> | SDAP UL Control PDU | 2 | P |

Table 7.1.4.2.3.2-3: Void

7.1.4.2.3.3	Specific message contents

Table 7.1.4.2.3.3-1: RadioBearerConfig-DRB (Preamble)

| Derivation Path: TS 38.508-1 [4], table 4.6.3-132 and condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF SEQUENCE { | 2 entries |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config SEQUENCE { |  |  |  |
| pdu-Session | The same as the PDU session ID in PDU SESSION ESTABLISHMENT REQUEST |  |  |
| sdap-HeaderDL | absent |  |  |
| sdap-HeaderUL | absent |  |  |
| defaultDRB | false |  |  |
| } |  |  |  |
| } |  |  |  |
| drb-Identity | k | k is allocated according to internal TTCN mapping |  |
| } |  |  |  |
| DRB-ToAddMod[2] SEQUENCE { |  | entry 2 |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config SEQUENCE { |  |  |  |
| pdu-Session | The same as the PDU session ID in PDU SESSION ESTABLISHMENT REQUEST |  |  |
| sdap-HeaderDL | absent |  |  |
| sdap-HeaderUL | present |  |  |
| defaultDRB | true |  |  |
| } |  |  |  |
| } |  |  |  |
| drb-Identity | j | j is allocated according to internal TTCN mapping |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.4.2.3.3-2: RadioBearerConfig-DRB (step 5, Table 7.1.4.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], table 4.6.3-132 and condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF SEQUENCE { | 2 entries |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config SEQUENCE { |  |  |  |
| pdu-Session | The same as the PDU session ID in PDU SESSION ESTABLISHMENT REQUEST |  |  |
| sdap-HeaderDL | absent |  |  |
| sdap-HeaderUL | absent |  |  |
| defaultDRB | false |  |  |
| mappedQoS-FlowsToAdd SEQUENCE { |  |  |  |
| QFI | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| drb-Identity | k | k is allocated according to internal TTCN mapping |  |
| } |  |  |  |
| DRB-ToAddMod[2] SEQUENCE { |  | entry 2 |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config SEQUENCE { |  |  |  |
| pdu-Session | The same as the PDU session ID in PDU SESSION ESTABLISHMENT REQUEST |  |  |
| sdap-HeaderDL | absent |  |  |
| sdap-HeaderUL | present |  |  |
| defaultDRB | true |  |  |
| mappedQoS-FlowsToAdd | Not present | The 'mappedQoS-Flows' is empty for the DRB. |  |
| } |  |  |  |
| } |  |  |  |
| drb-Identity | j | j is allocated according to internal TTCN mapping |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.4.2.3.3-3: PDU SESSION ESTABLISHMENT ACCEPT (Preamble)

| Derivation Path: TS 38.508-1 [4], table 4.7.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDU session ID | The same as the PDU session ID in PDU SESSION ESTABLISHMENT REQUEST |  |  |
| Authorized QoS rules | 3 entries |  |  |
| QoS rule [1] | Reference QoS rule #1 as defined in Table 4.8.2.1-1 | QFI=1 |  |
| QoS rule [2] | Reference QoS rule #4 as defined in Table 4.8.2.1-4 except DQR bit set to ‘0’ | QFI=2 |  |
| QoS rule [3] | Reference QoS rule #5 as defined in Table 4.8.2.1-5 | QFI=5 |  |
| Mapped EPS bearer contexts | Not Present |  |  |
| Authorized QoS flow descriptions | 3 entries |  |  |
| QoS flow [1] | Reference QoS flow #1 as defined in Table 7.1.4.2.3.3-4 | QFI=1 |  |
| QoS flow [2] | Reference QoS flow #2 as defined in Table 7.1.4.2.3.3-5 | QFI=2 |  |
| QoS flow [3] | Reference QoS flow #3 as defined in Table 4.8.2.3-3 | QFI=5 |  |

Table 7.1.4.2.3.3-4: Reference QoS flow #1 (Preamble)

| Derivation Path: TS 38.508-1 [4], table 4.8.2.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description | 1 entry |  |  |
| QFI | ‘00 0001’B | QFI 1 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| 5QI | ‘0000 1001’B | 5QI 9 |  |

Table 7.1.4.2.3.3-5: Reference QoS flow #2 (Preamble)

| Derivation Path: TS 38.508-1 [4], table 4.8.2.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description | 1 entry |  |  |
| QFI | ‘00 0010’B | QFI 2 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| 5QI | ‘0000 0101’B | 5QI 5 |  |