### 8.1.5	RRC others

#### 8.1.5.1	UE capability transfer

##### 8.1.5.1.1	UE capability transfer / Success

8.1.5.1.1.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE receives an UECapabilityEnquiry message }

then { UE transmits an UECapabilityInformation message including UE radio access capability information corresponding to the ue-CapabilityRequest variable }

}

8.1.5.1.1.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.331, clauses 5.6.1.3, 5.6.1.4, 5.7.7.2 and 5.7.7.3. Unless otherwise stated these are Rel-15 requirements.

[TS 38.331, clause 5.6.1.3]

The UE shall set the contents of UECapabilityInformation message as follows:

1>	if the ue-CapabilityRAT-RequestList contains a UE-CapabilityRAT-Request with rat-Type set to nr:

2>	include in the ue-CapabilityRAT-ContainerList a UE-CapabilityRAT-Container of the type UE-NR-Capability and with the rat-Type set to nr;

2>	include the supportedBandCombinationList, featureSets and featureSetCombinations as specified in clause 5.6.1.4;

…

1>	submit the UECapabilityInformation message to lower layers for transmission, upon which the procedure ends.

[TS 38.331, clause 5.6.1.4]

The UE invokes the procedures in this clause if the NR or E-UTRA network requests UE capabilities for nr, eutra-nr or eutra. This procedure is invoked once per requested rat-Type (see clause 5.6.1.3 for capability enquiry by the NR network; see TS 36.331 [10], clause 5.6.3.3 for capability enquiry by the E-UTRA network). The UE shall ensure that the feature set IDs are consistent across feature sets, feature set combinations and band combinations in all three UE capability containers that the network queries with the same fields with the same values, i.e. UE-CapabilityRequestFilterNR and fields in UECapabilityEnquiry message (i.e. requestedFreqBandsNR-MRDC, requestedCapabilityNR and eutra-nr-only flag) as defined in TS 36.331, where applicable.

NOTE 1:	Capability enquiry without frequencyBandListFilter is not supported.

[TS 38.331, clause 5.7.7.2]

A UE capable of UL RRC message segmentation in RRC_CONNECTED will initiate the procedure when the following conditions are met:

1>	if the RRC message segmentation is enabled based on the field rrc-SegAllowed received, and

1>	if the encoded RRC message is larger than the maximum supported size of a PDCP SDU specified in TS 38.323 [5];

Upon initiating the procedure, the UE shall:

1>	initiate transmission of the ULDedicatedMessageSegment message as specified in 5.7.7.3;

[TS 38.331, clause 5.7.7.3]

The UE shall segment the encoded RRC PDU based on the maximum supported size of a PDCP SDU specified in TS 38.323 [5]. UE shall minimize the number of segments and set the contents of the ULDedicatedMessageSegment messages as follows:

1>	F or each new UL DCCH message, set the segmentNumber to 0 for the first message segment and increment the segmentNumber for each subsequent RRC message segment;

1>	set rrc-MessageSegmentContainer to include the segment of the UL DCCH message corresponding to the segmentNumber;

1>	if the segment included in the rrc-MessageSegmentContainer is the last segment of the UL DCCH message:

2>	set the rrc-MessageSegmentType to lastSegment;

1>	else:

2>	set the rrc-MessageSegmentType to notLastSegment;

1>	submit all the ULDedicatedMessageSegment messages generated for the segmented RRC message to lower layers for transmission in ascending order based on the segmentNumber, upon which the procedure ends.

8.1.5.1.1.3	Test Description

8.1.5.1.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used in NR cell.

UE:

-	None.

Preamble:

-	The UE is in 5GS state 3N-A according to TS 38.508-1 [4], clause 4.4A on NR Cell 1.

8.1.5.1.1.3.2	Test procedure sequence

Table 8.1.5.1.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a UECapabilityEnquiry message including nr and rrc-segAllowed-r16 set to enabled. | <-- | UECapabilityEnquiry | - | - |
| - | EXCEPTION: The "lower case letter" identifies a step sequence that takes place depending on the size of UE capability message. IF pc_NR_UL_Segmentation and UE’s encoded UECapabilityInformation message >= Max PDCP SDU size then steps 2a1 and 2a2 will be performed else step 2b1 will be performed. | - | - | - | - |
| - | EXCEPTION: Step 2a1 is repeated a maximum of 16 times or till rrc-MessageSegmentType-16 IE within the UlDedicatedMessageSegment message indicates rrc-MessageSegmentType-r16 = lastSegment. | - | - | - | - |
| 2a1 | The UE sends segments of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment message. | --> | ULDedicatedMessageSegment | - | - |
| 2a2 | Check: Is the received UE radio access capability information as per the ue-CapabilityRequest variable? | - | - | 1 | P |
| 2b1 | Check: Does the UE transmit a UECapabilityInformation message including UE radio access capability information as per the ue-CapabilityRequest variable? | --> | UECapabilityInformation | 1 | P |

8.1.5.1.1.3.3	Specific message contents

Table 8.1.5.1.1.3.3-1: UECapabilityEnquiry (step 1, Table 8.1.5.1.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-31 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UECapabilityEnquiry ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueCapabilityEnquiry SEQUENCE { |  |  |  |
| ue-CapabilityRAT-RequestList SEQUENCE (SIZE (1.. maxRAT-CapabilityContainers)) OF CapabilityRAT-Request { | - |  |  |
| UE-CapabilityRAT-Request[1] SEQUENCE { |  | entry 1 |  |
| rat-Type | nr |  |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| ue-CapabilityEnquiryExt | OCTET STRING Containing UECapabilityEnquiry-v1560-IEs (Table 8.1.5.1.1.3.3-4) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.1.1.3.3-2: UECapabilityInformation (steps 2a2 and 2b1, Table 8.1.5.1.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UECapabilityInformation ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueCapabilityInformation SEQUENCE { |  |  |  |
| ue-CapabilityRAT-ContainerList SEQUENCE (SIZE (0..maxRAT-CapabilityContainers)) OF CapabilityRAT-Request { |  |  |  |
| UE-CapabilityRAT-Request[1] SEQUENCE { |  | entry 1 |  |
| rat-Type | nr |  |  |
| ueCapabilityRAT-Container | UE-NR-Capability | Encoded as per TS 38.331 [12] clause 5.6.1 |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.1.1.3.3-3: UE-NR-Capability (Table 8.1.5.1.1.3.3-2)

| Derivation Path: TS 38.331 [12], clause 6.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE-NR-Capability ::= SEQUENCE { |  |  |  |
| accessStratumRelease | Checked | "rel-15" or higher |  |
|  | Checked | "rel-17" or higher | pc_supportOfRedCap_r17 |
| pdcp-Parameters SEQUENCE { |  |  |  |
| supportedROHC-Profiles SEQUENCE { |  |  |  |
| profile0x0000 | Not checked |  |  |
| profile0x0001 | Not checked |  |  |
| profile0x0002 | Not checked |  |  |
| profile0x0003 | Not checked |  |  |
| profile0x0004 | Not checked |  |  |
| profile0x0006 | Not checked |  |  |
| profile0x0101 | Not checked |  |  |
| profile0x0102 | Not checked |  |  |
| profile0x0103 | Not checked |  |  |
| profile0x0104 | Not checked |  |  |
| } |  |  |  |
| maxNumberROHC-ContextSessions | Not checked |  |  |
| uplinkOnlyROHC-Profiles | Not checked |  |  |
| continueROHC-Context | Not checked |  |  |
| outOfOrderDelivery | Checked |  | pc_outOfOrderDelivery |
| shortSN | Checked |  | pc_shortSN |
| pdcp-DuplicationSRB | Checked |  | pc_pdcp_DuplicationSplitSRB |
|  | Not Present |  | pc_supportOfRedCap_r17 |
| pdcp-DuplicationMCG-OrSCG-DRB | Checked |  | pc_pdcp_DuplicationMCG_OrSCG_DRB |
|  | Not Present |  | pc_supportOfRedCap_r17 |
| drb-IAB-r16 | Not checked |  |  |
| non-DRB-IAB-r16 | Not checked |  |  |
| extendedDiscardTimer-r16 | Not checked |  |  |
| continueEHC-Context-r16 | Not checked |  |  |
| ehc-r16 | Checked |  | pc_NR_ehc_r16 |
| maxNumberEHC-Contexts-r16 | Not checked |  |  |
| jointEHC-ROHC-Config-r16 | Not checked |  |  |
| pdcp-DuplicationMoreThanTwoRLC-r16 | Checked |  | pc_pdcp_DuplicationMoreThanTwoRLC_r16 |
| longSN-RedCap-r17 | Checked |  | pc_supportOfRedCap_r17 and pc_LongSN |
| udc-r17 SEQUENCE { | Checked | TS 38.306 cl. 4.2.4. Indicates whether the UE supports the uplink data compression operation as specified in TS 38.323. Nested parameters apply on presence of udc-r17 | pc_NR_udc_r17 |
| standardDictionary-r17 | Checked |  | pc_NR_udc_standardDictionary_r17 |
| operatorDictionary-r17 | Not checked |  |  |
| continueUDC-r17 | Checked |  | pc_NR_udc_continueUDC_r17 |
| supportOfBufferSize-r17 | Not checked |  |  |
| } |  |  |  |
| longSN-NCR-r18 | Not checked |  |  |
| supportOfPDU-SetDiscard-r18 | Not checked |  |  |
| psi-BasedDiscard-r18 | Not checked |  |  |
| supportOfSN-GapReport-r18 | Checked |  | pc_supportOfSN_GapReport_r18 |
| } |  |  |  |
| rlc-Parameters SEQUENCE { |  |  |  |
| am-WithShortSN | Checked |  | pc_am_WithShortSN |
| um-WithShortSN | Checked |  | pc_um_WithShortSN |
| um-WithLongSN | Checked |  | pc_um_WIthLongSN |
| am-WithLongSN-RedCap-r17 | Checked |  | pc_supportOfRedCap_r17 and pc_am_WithLongSN |
| } |  |  |  |
| mac-Parameters SEQUENCE { |  |  |  |
| mac-ParametersCommon SEQUENCE { |  |  |  |
| lcp-Restriction | Checked |  | pc_lcp_Restriction |
| dummy | Not checked |  |  |
| lch-ToSCellRestriction | Not checked |  |  |
| recommendedBitRate | Checked |  | pc_recommendedBitRate |
| recommendedBitRateQuery | Checked |  | pc_recommendedBitRateQuery |
| recommendedBitRateMultiplier-r16 | Not checked |  |  |
| preEmptiveBSR-r16 | Not checked |  |  |
| autonomousTransmission-r16 | Not checked |  |  |
| lch-PriorityBasedPrioritization-r16 | Checked |  | pc_lch_PriorityBasedPrioritization_r16 |
| lch-ToConfiguredGrantMapping-r16 | Not checked |  |  |
| lch-ToGrantPriorityRestriction-r16 | Checked |  | pc_lch_ToGrantPriorityRestriction_r16 |
| singlePHR-P-r16 | Not checked |  |  |
| ul-LBT-FailureDetectionRecovery-r16 | Checked |  | pc_UL_LBT_FailureDetectionRecovery_r16 |
| tdd-MPE-P-MPR-Reporting-r16 | Not checked |  |  |
| lcid-ExtensionIAB-r16 | Not checked |  |  |
| spCell-BFR-CBRA-r16 | Not checked |  |  |
| srs-ResourceId-Ext-r16 | Not checked |  |  |
| enhancedUuDRX-forSidelink-r17 | Not checked |  |  |
| mg-ActivationRequestPRS-Meas-r17 | Not checked |  |  |
| mg-ActivationCommPRS-Meas-r17 | Not checked |  |  |
| intraCG-Prioritization-r17 | Not checked |  |  |
| jointPrioritizationCG-Retx-Timer-r17 | Not checked |  |  |
| survivalTime-r17 | Checked |  | pc_survivalTime_r17 |
| lcg-ExtensionIAB-r17 | Not checked |  |  |
| harq-FeedbackDisabled-r17 | Checked |  | pc_harq_FeedbackDisabled_r17 |
| uplink-Harq-ModeB-r17 | Checked |  | pc_uplink_Harq_ModeB_r17 |
| sr-TriggeredBy-TA-Report-r17 | Not checked |  |  |
| extendedDRX-CycleInactive-r17 | Checked |  | pc_extendedDRX_CycleInactive_r17 |
| simultaneousSR-PUSCH-DiffPUCCH-groups-r17 | Not checked |  |  |
| lastTransmissionUL-r17 | Not checked |  |  |
| harq-RTT-TimerDL-ForNTN-MulticastMBS-r17 | Not checked |  |  |
| sr-TriggeredByTA-ReportATG-r18 | Not checked |  |  |
| extendedDRX-CycleInactive-r18 | Checked |  | pc_extendedDRX_CycleInactive_r18 |
| additionalBS-Table-r18 | Not checked |  |  |
| delayStatusReport-r18 | Not checked |  |  |
| cg-RetransmissionMonitoringDisabling-r18 | Not checked |  |  |
| non-IntegerDRX-r18 | eChecked |  | pc_non_IntegerDRX_r18 |
| } |  |  |  |
| mac-ParametersXDD-Diff SEQUENCE { |  |  |  |
| skipUplinkTxDynamic | Checked (NOTE 4) |  | pc_skipUplinkTxDynamic |
| logicalChannelSR-DelayTimer | Checked (NOTE 5) |  | pc_logicalChannelSR_DelayTimer |
| longDRX-Cycle | Checked (NOTE 6) |  | pc_longDRX_Cycle |
| shortDRX-Cycle | Checked (NOTE 7) |  | pc_shortDRX_Cycle |
| multipleSR-Configurations | Not checked |  |  |
| multipleConfiguredGrants | Checked (NOTE 12) |  | pc_multipleConfiguredGrants |
| } |  |  |  |
| } |  |  |  |
| phy-Parameters SEQUENCE { |  |  |  |
| phy-ParametersCommon SEQUENCE { |  |  |  |
| csi-RS-CFRA-ForHO | Checked |  | pc_csi_RS_CFRA_ForHO |
| dynamicPRB-BundlingDL | Not checked |  |  |
| sp-CSI-ReportPUCCH | Not checked |  |  |
| sp-CSI-ReportPUSCH | Not checked |  |  |
| nzp-CSI-RS-IntefMgmt | Not checked |  |  |
| type2-SP-CSI-Feedback-LongPUCCH | Not checked |  |  |
| precoderGranularityCORESET | Not checked |  |  |
| dynamicHARQ-ACK-Codebook | supported |  |  |
| semiStaticHARQ-ACK-Codebook | Not checked |  |  |
| spatialBundlingHARQ-ACK | Checked |  | pc_spatialBundlingHARQ_ACK |
| dynamicBetaOffsetInd-HARQ-ACK-CSI | Not checked |  |  |
| pucch-Repetition-F1-3-4 | Not checked |  |  |
| ra-Type0-PUSCH | Checked |  | pc_ra_Type0_PUSCH |
| dynamicSwitchRA-Type0-1-PDSCH | Checked |  | pc_dynamicSwitchRA_Type0_1_PDSCH |
| dynamicSwitchRA-Type0-1-PUSCH | Checked |  | pc_dynamicSwitchRA_Type0_1_PUSCH |
| pdsch-MappingTypeA | supported |  |  |
| pdsch-MappingTypeB | Checked |  | pc_pdsch_MappingTypeB |
| interleavingVRB-ToPRB-PDSCH | Checked |  | pc_interleavingVRB_ToPRB_PDSCH |
| interSlotFreqHopping-PUSCH | Not checked |  |  |
| type1-PUSCH-RepetitionMultiSlots | Checked |  | pc_type1_PUSCH_RepetitionMultiSlots |
| type2-PUSCH-RepetitionMultiSlots | Checked |  | pc_type2_PUSCH_RepetitionMultiSlots |
| pusch-RepetitionMultiSlots | Checked |  | pc_pusch_RepetitionMultiSlots |
| pdsch-RepetitionMultiSlots | Checked |  | pc_pdsch_RepetitionMultiSlots |
| downlinkSPS | Checked |  | pc_downlinkSPS |
| configuredUL-GrantType1 | Checked |  | pc_configuredUL_GrantType1 |
| configuredUL-GrantType2 | Checked |  | pc_configuredUL_GrantType2 |
| pre-EmptIndication-DL | Not checked |  |  |
| cbg-TransIndication-DL | Not checked |  |  |
| cbg-TransIndication-UL | Not checked |  |  |
| cbg-FlushIndication-DL | Not checked |  |  |
| dynamicHARQ-ACK-CodeB-CBG-Retx-DL | Not checked |  |  |
| rateMatchingResrcSetSemi-Static | Not checked |  |  |
| rateMatchingResrcSetDynamic | Not checked |  |  |
| bwp-SwitchingDelay | Checked | TS 38.306 [23] clause 4.2.7.10: It is mandatory to report type 1 or type 2. This capability is not applicable to IAB-MT | pc_bwp_SwitchingDelay_Type1 or pc_bwp_SwitchingDelay_Type2 |
| twoStepRACH-r16 | Checked |  | pc_twoStepRACH_r16 |
| dci-Format1-2And0-2-r16 | Checked |  | pc_dci_Format1_2And0_2_r16 |
| monitoringDCI-SameSearchSpace-r16 | Not checked |  |  |
| type2-CG-ReleaseDCI-0-1-r16 | Not checked |  |  |
| type2-CG-ReleaseDCI-0-2-r16 | Not checked |  |  |
| sps-ReleaseDCI-1-1-r16 | Not checked |  |  |
| sps-ReleaseDCI-1-2-r16 | Not checked |  |  |
| csi-TriggerStateNon-ActiveBWP-r16 | Not checked |  |  |
| separateSMTC-InterIAB-Support-r16 | Not checked |  |  |
| separateRACH-IAB-Support-r16 | Not checked |  |  |
| ul-flexibleDL-SlotFormatSemiStatic-IAB-r16 | Not checked |  |  |
| ul-flexibleDL-SlotFormatDynamics-IAB-r16 | Not checked |  |  |
| dft-S-OFDM-WaveformUL-IAB-r16 | Not checked |  |  |
| dci-25-AI-RNTI-Support-IAB-r16 | Not checked |  |  |
| t-DeltaReceptionSupport-IAB-r16 | Not checked |  |  |
| guardSymbolReportReception-IAB-r16 | Not checked |  |  |
| harqACK-CB-SpatialBundlingPUCCH-Group-r16 | Not checked |  |  |
| crossSlotScheduling-r16 | Not checked |  |  |
| maxNumberSRS-PosPathLossEstimateAllServingCells-r16 | Not checked |  |  |
| extendedCG-Periodicities-r16 | Not checked |  |  |
| extendedSPS-Periodicities-r16 | Not checked |  |  |
| codebookVariantsList-r16 | Not checked |  |  |
| pusch-RepetitionTypeA-r16 | Checked |  | pc_pusch_RepetitionTypeA_r16 |
| dci-DL-PriorityIndicator-r16 | Checked |  | pc_dci_DL_PriorityIndicator_r16 |
| dci-UL-PriorityIndicator-r16 | Checked |  | pc_dci_UL_PriorityIndicator_r16 |
| maxNumberPathlossRS-Update-r16 | Not checked |  |  |
| type2-HARQ-ACK-Codebook-r16 | Not checked |  |  |
| maxTotalResourcesForAcrossFreqRanges-r16 | Not checked |  |  |
| harqACK-separateMultiDCI-MultiTRP-r16 | Not checked |  |  |
| harqACK-jointMultiDCI-MultiTRP-r16 | Not checked |  |  |
| bwp-SwitchingMultiCCs-r16 | Not checked |  |  |
| guardSymbolReportReception-IAB-r17 | Not checked |  |  |
| restricted-IAB-DU-BeamReception-r17 | Not checked |  |  |
| recommended-IAB-MT-BeamTransmission-r17 | Not checked |  |  |
| case6-TimingAlignmentReception-IAB-r17 | Not checked |  |  |
| case7-TimingAlignmentReception-IAB-r17 | Not checked |  |  |
| dl-tx-PowerAdjustment-IAB-r17 | Not checked |  |  |
| desired-ul-tx-PowerAdjustment-r17 | Not checked |  |  |
| fdm-SoftResourceAvailability-DynamicIndication-r17 | Not checked |  |  |
| updated-T-DeltaRangeReception-r17 | Not checked |  |  |
| slotBasedDynamicPUCCH-Rep-r17 | Checked |  | pc_pucch_Repetition_F0_1_2_3_4_DynamicIndication_r17 |
| sps-HARQ-ACK-Deferral-r17 | Not checked |  |  |
| unifiedJointTCI-commonUpdate-r17 | Not checked |  |  |
| mTRP-PDCCH-singleSpan-r17 | Not checked |  |  |
| supportedActivatedPRS-ProcessingWindow-r17 | Not checked |  |  |
| cg-TimeDomainAllocationExtension-r17 | Not checked |  |  |
| } |  |  |  |
| phy-ParametersXDD-Diff SEQUENCE { |  |  |  |
| dynamicSFI | Not checked |  |  |
| twoPUCCH-F0-2-ConsecSymbols | Not checked |  |  |
| twoDifferentTPC-Loop-PUSCH | Not checked |  |  |
| twoDifferentTPC-Loop-PUCCH | Not checked |  |  |
| dl-SchedulingOffset-PDSCH-TypeA | Checked (NOTE 17) |  | pc_dl_SchedulingOffset_PDSCH_TypeA |
| dl-SchedulingOffset-PDSCH-TypeB | Not checked |  |  |
| ul-SchedulingOffset | Not checked |  |  |
| } |  |  |  |
| phy-ParametersFRX-Diff SEQUENCE { |  |  |  |
| dynamicSFI | Not checked |  |  |
| dummy1 | Not checked |  |  |
| twoFL-DMRS | Not checked |  |  |
| dummy2 | Not checked |  |  |
| dummy3 | Not checked |  |  |
| supportedDMRS-TypeDL | Not checked |  |  |
| supportedDMRS-TypeUL | Not checked |  |  |
| semiOpenLoopCSI | Not checked |  |  |
| csi-ReportWithoutPMI | Not checked |  |  |
| csi-ReportWithoutCQI | Not checked |  |  |
| onePortsPTRS | Not checked |  |  |
| twoPUCCH-F0-2-ConsecSymbols | Not checked |  |  |
| pucch-F2-WithFH | supported |  |  |
| pucch-F3-WithFH | supported |  |  |
| pucch-F4-WithFH | Not checked |  |  |
| pucch-F0-2WithoutFH | Not checked |  |  |
| pucch-F1-3-4WithoutFH | Not checked |  |  |
| mux-SR-HARQ-ACK-CSI-PUCCH-MultiPerSlot | Not checked |  |  |
| uci-CodeBlockSegmentation | Not checked |  |  |
| onePUCCH-LongAndShortFormat | Not checked |  |  |
| twoPUCCH-AnyOthersInSlot | Checked (NOTE 26) |  | pc_twoPUCCH_AnyOthersInSlot |
| intraSlotFreqHopping-PUSCH | Not checked |  |  |
| pusch-LBRM | Not checked |  |  |
| pdcch-BlindDetectionCA | Not checked |  |  |
| tpc-PUSCH-RNTI | Not checked |  |  |
| tpc-PUCCH-RNTI | Not checked |  |  |
| tpc-SRS-RNTI | Not checked |  |  |
| absoluteTPC-Command | Not checked |  |  |
| twoDifferentTPC-Loop-PUSCH | Not checked |  |  |
| twoDifferentTPC-Loop-PUCCH | Not checked |  |  |
| pusch-HalfPi-BPSK | Not checked |  |  |
| pucch-F3-4-HalfPi-BPSK | Not checked |  |  |
| almostContiguousCP-OFDM-UL | Not checked |  |  |
| sp-CSI-RS | Not checked |  |  |
| sp-CSI-IM | Not checked |  |  |
| tdd-MultiDL-UL-SwitchPerSlot | Not checked |  |  |
| multipleCORESET | Not checked |  |  |
| csi-RS-IM-ReceptionForFeedback | Not checked | CSI-RS-IM-ReceptionForFeedback |  |
| csi-RS-ProcFrameworkForSRS | Not checked | CSI-RS-ProcFrameworkForSRS |  |
| csi-ReportFramework | Not checked | CSI-ReportFramework |  |
| mux-SR-HARQ-ACK-CSI-PUCCH-OncePerSlot SEQUENCE { |  |  |  |
| sameSymbol | Not checked |  |  |
| diffSymbol | Not checked |  |  |
| } |  |  |  |
| mux-SR-HARQ-ACK-PUCCH | Not checked |  |  |
| mux-MultipleGroupCtrlCH-Overlap | Not checked |  |  |
| dl-SchedulingOffset-PDSCH-TypeA | Checked (NOTE 18) |  | pc_dl_SchedulingOffset_PDSCH_TypeA |
| dl-SchedulingOffset-PDSCH-TypeB | Not checked |  |  |
| ul-SchedulingOffset | Not checked |  |  |
| dl-64QAM-MCS-TableAlt | Not checked |  |  |
| ul-64QAM-MCS-TableAlt | Not checked |  |  |
| cqi-TableAlt | Not checked |  |  |
| oneFL-DMRS-TwoAdditionalDMRS-UL | Not checked |  |  |
| twoFL-DMRS-TwoAdditionalDMRS-UL | Not checked |  |  |
| oneFL-DMRS-ThreeAdditionalDMRS-UL | Not checked |  |  |
| } |  |  |  |
| phy-ParametersFR1 SEQUENCE { |  |  |  |
| pdcchMonitoringSingleOccasion | Not checked |  |  |
| scs-60kHz | Not checked |  |  |
| pdsch-256QAM-FR1 | Checked |  | pc_pdsch_256QAM_FR1 |
| pdsch-RE-MappingFR1-PerSymbol | Not checked |  |  |
| k1-RangeExtensionATG-r18 | Not checked |  |  |
| maxHARQ-ProcessNumberATG-r18 | Not checked |  |  |
| uplinkPreCompensationATG-r18 | Not checked |  |  |
| uplinkTA-ReportingATG-r18 | Checked |  | pc_uplinkTA_ReportingATG_r18 |
| advReceiver-MU-MIMO-r18 | Not checked |  |  |
| deltaPowerClassReporting-r18 | Checked |  | pc_deltaPowerClassReporting_r18 |
| support12PRB-CORESET0-GSCN-41637-r18 | Not checked |  |  |
| support5MHz-ChannelBW-20PRB-CORESET0-r18 | Not checked |  |  |
| } |  |  |  |
| phy-ParametersFR2 SEQUENCE { |  |  |  |
| dummy | Not checked |  |  |
| pdsch-RE-MappingFR2-PerSymbol | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| rf-Parameters SEQUENCE { |  |  |  |
| supportedBandListNR SEQUENCE (SIZE (1..maxBands)) OF BandNR { |  |  |  |
| BandNR[i] SEQUENCE { |  |  |  |
| bandNR | Checked | Checked for 'maxBands' entries of FreqBandIndicatorNR[i] | pc_nrBandx ('x' being the band number/type related PICS listed in TS 38.508-2 [5]) |
| modifiedMPR-Behaviour | Not checked |  |  |
| mimo-ParametersPerBand | Checked | MIMO-ParametersPerBand[i] (Table 8.1.5.1.1.3.3-12) |  |
| extendedCP | Not checked |  |  |
| multipleTCI | supported |  |  |
| bwp-WithoutRestriction | Checked |  | pc_bwp_WithoutRestriction |
| bwp-SameNumerology | Checked (NOTE 21) |  | pc_bwp_SameNumerology_upto2_FR1_FDD or pc_bwp_SameNumerology_upto2_FR1_TDD or pc_bwp_SameNumerology_upto2_FR2 or pc_bwp_SameNumerology_upto4_FR1_FDD or pc_bwp_SameNumerology_upto4_FR1_TDD or pc_bwp_SameNumerology_upto4_FR2 |
| bwp-DiffNumerology | upto4 (NOTE 20) |  | pc_bwp_DiffNumerology_FR1_FDD or pc_bwp_DiffNumerology_FR1_TDD or pc_bwp_DiffNumerology_FR2 |
| crossCarrierScheduling-SameSCS | Not checked |  |  |
| pdsch-256QAM-FR2 | Checked (NOTE 8) |  | pc_pdsch_256QAM_FR2 |
| pusch-256QAM | Checked (NOTE 9) |  | pc_pusch_256QAM_FR1 OR pc_pusch_256QAM_FR2 |
| ue-PowerClass | Not checked |  |  |
| rateMatchingLTE-CRS | Not checked |  |  |
| channelBWs-DL CHOICE { |  |  |  |
| fr1 SEQUENCE { |  |  |  |
| scs-15kHz | Not checked |  |  |
| scs-30kHz | Not checked |  |  |
| scs-60kHz | Not checked |  |  |
| } |  |  |  |
| fr2 SEQUENCE { |  |  |  |
| scs-60kHz | Not checked |  |  |
| scs-120kHz | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| channelBWs-UL CHOICE { |  |  |  |
| fr1 SEQUENCE { |  |  |  |
| scs-15kHz | Not checked |  |  |
| scs-30kHz | Not checked |  |  |
| scs-60kHz | Not checked |  |  |
| } |  |  |  |
| fr2 SEQUENCE { |  |  |  |
| scs-60kHz | Not checked |  |  |
| scs-120kHz | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| sharedSpectrumChAccessParamsPerBand-r16 | Checked | SharedSpectrumChAccessParamsPerBand-r16 (Table 8.1.5.1.1.3.3-13) |  |
| cancelOverlappingPUSCH-r16 | Not checked |  |  |
| multipleRateMatchingEUTRA-CRS-r16 | Not checked |  |  |
| overlapRateMatchingEUTRA-CRS-r16 | Not checked |  |  |
| pdsch-MappingTypeB-Alt-r16 | Not checked |  |  |
| oneSlotPeriodicTRS-r16 | Not checked |  |  |
| olpc-SRS-Pos-r16 | Not checked |  |  |
| spatialRelationsSRS-Pos-r16 | Not checked |  |  |
| simulSRS-MIMO-TransWithinBand-r16 | Not checked |  |  |
| channelBW-DL-IAB-r16 | Not checked |  |  |
| channelBW-UL-IAB-r16 | Not checked |  |  |
| rasterShift7dot5-IAB-r16 | Not checked |  |  |
| ue-PowerClass-v1610 | Not checked |  |  |
| condHandover-r16 | Checked |  | pc_condHandover_r16 |
| condHandoverFailure-r16 | Checked |  | pc_condHandoverFailure_r16 |
| condHandoverTwoTriggerEvents-r16 | Checked |  | pc_condHandoverTwoTriggerEvents_r16 |
| condPSCellChange-r16 | Checked |  | pc_condPSCellChange_r16 |
| condPSCellChangeTwoTriggerEvents-r16 | Not checked |  |  |
| mpr-PowerBoost-FR2-r16 | Not checked |  |  |
| activeConfiguredGrant-r16 | Checked |  | pc_activeConfiguredGrant_r16 |
| jointReleaseConfiguredGrantType2-r16 | Not checked |  |  |
| sps-r16 | Checked |  | pc_multi_sps_r16 |
| jointReleaseSPS-r16 | Not checked |  |  |
| simulSRS-TransWithinBand-r16 | Not checked |  |  |
| trs-AdditionalBandwidth-r16 | Not checked |  |  |
| handoverIntraF-IAB-r16 | Not checked |  |  |
| maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16 | Checked |  | pc_nrBandx_maxUplinkDutyCycle_PC1dot5_MPE_FR1_r16 (‘x’ being the band number related PICS listed in Table A.4.3.1-4h of TS 38.508-2 [2]) |
| txDiversity-r16 | Checked |  | pc_txDiversity_r16 |
| pdsch-1024QAM-FR1-r17 | Not checked |  |  |
| ue-PowerClass-v1700 | Not checked |  |  |
| fr2-2-AccessParamsPerBand-r17 | Not checked |  |  |
| rlm-Relaxation-r17 | Not checked |  |  |
| bfd-Relaxation-r17 | Not checked |  |  |
| cg-SDT-r17 | Checked |  | pc_cg_SDT_r17 |
| locationBasedCondHandover-r17 | Checked |  | pc_locationBasedCondHandover_r17 |
| timeBasedCondHandover-r17 | Checked |  | pc_timeBasedCondHandover_r17 |
| eventA4BasedCondHandover-r17 | Checked |  | pc_eventA4BasedCondHandover_r17 |
| mn-InitiatedCondPSCellChangeNRDC-r17 | Not checked |  |  |
| sn-InitiatedCondPSCellChangeNRDC-r17 | Checked |  | pc_sn_InitiatedCondPSCellChangeNRDC_r17 |
| pdcch-SkippingWithoutSSSG-r17 | Not checked |  |  |
| sssg-Switching-1BitInd-r17 | Checked |  | pc_sssg_Switching_1BitInd_r17 |
| sssg-Switching-2BitInd-r17 | Checked |  | pc_sssg_Switching_2BitInd_r17 |
| pdcch-SkippingWithSSSG-r17 | Not checked |  |  |
| searchSpaceSetGrp-switchCap2-r17 | Checked |  | pc_searchSpaceSetGrp_switchCap2_r17 |
| uplinkPreCompensation-r17 | Not checked |  |  |
| uplink-TA-Reporting-r17 | Checked |  | pc_uplink_TA_Reporting_r17 |
| max-HARQ-ProcessNumber-r17 | Not checked |  |  |
| type2-HARQ-Codebook-r17 | Not checked |  |  |
| type1-HARQ-Codebook-r17 | Not checked |  |  |
| type3-HARQ-Codebook-r17 | Not checked |  |  |
| ue-specific-K-Offset-r17 | Checked |  | pc_ue_specific_K_Offset_r17 |
| multiPDSCH-SingleDCI-FR2-1-SCS-120kHz-r17 | Not checked |  |  |
| multiPUSCH-SingleDCI-FR2-1-SCS-120kHz-r17 | Not checked |  |  |
| parallelPRS-MeasRRC-Inactive-r17 | Not checked |  |  |
| nr-UE-TxTEG-ID-MaxSupport-r17 | Not checked |  |  |
| prs-ProcessingRRC-Inactive-r17 | Not checked |  |  |
| prs-ProcessingWindowType1A-r17 | Not checked |  |  |
| prs-ProcessingWindowType1B-r17 | Not checked |  |  |
| prs-ProcessingWindowType2-r17 | Not checked |  |  |
| srs-AllPosResourcesRRC-Inactive-r17 | Not checked |  |  |
| olpc-SRS-PosRRC-Inactive-r17 | Not checked |  |  |
| spatialRelationsSRS-PosRRC-Inactive-r17 | Not checked |  |  |
| maxNumberPUSCH-TypeA-Repetition-r17 | Checked |  | pc_maxNumberPUSCH_TypeA_Repetition_r17 |
| puschTypeA-RepetitionsAvailSlot-r17 | Checked |  | pc_puschTypeA_RepetitionsAvailSlot_r17 |
| tb-ProcessingMultiSlotPUSCH-r17 | Checked |  | pc_tb_ProcessingMultiSlotPUSCH_r17 |
| tb-ProcessingRepMultiSlotPUSCH-r17 | Checked |  | pc_tb_ProcessingRepMultiSlotPUSCH_r17 |
| maxDurationDMRS-Bundling-r17 | Not checked |  |  |
| pusch-RepetitionMsg3-r17 | Checked |  | pc_pusch_RepetitionMsg3_r17 |
| sharedSpectrumChAccessParamsPerBand-v1710 | Not checked |  |  |
| parallelMeasurementWithoutRestriction-r17 | Not checked |  |  |
| maxNumber-NGSO-SatellitesWithinOneSMTC-r17 | Not checked |  |  |
| k1-RangeExtension-r17 | Not checked |  |  |
| aperiodicCSI-RS-FastScellActivation-r17 | Not checked |  |  |
| aperiodicCSI-RS-AdditionalBandwidth-r17 | Not checked |  |  |
| bwp-WithoutCD-SSB-OrNCD-SSB-RedCap-r17 | Checked |  | pc_bwp_WithoutCD_SSB_OrNCD_SSB_RedCap_r17 |
| halfDuplexFDD-TypeA-RedCap-r17 | Checked |  | pc_halfDuplexFDD_TypeA_RedCap_r17 |
| posSRS-RRC-Inactive-OutsideInitialUL-BWP-r17 | Not checked |  |  |
| channelBWs-DL-SCS-480kHz-FR2-2-r17 | Not checked |  |  |
| channelBWs-UL-SCS-480kHz-FR2-2-r17 | Not checked |  |  |
| channelBWs-DL-SCS-960kHz-FR2-2-r17 | Not checked |  |  |
| channelBWs-UL-SCS-960kHz-FR2-2-r17 | Not checked |  |  |
| ul-GapFR2-r17 | Not checked |  |  |
| oneShotHARQ-feedbackTriggeredByDCI-1-2-r17 | Not checked |  |  |
| oneShotHARQ-feedbackPhy-Priority-r17 | Not checked |  |  |
| enhancedType3-HARQ-CodebookFeedback-r17 | Not checked |  |  |
| triggeredHARQ-CodebookRetx-r17 | Not checked |  |  |
| ue-OneShotUL-TimingAdj-r17 | Not checked |  |  |
| pucch-Repetition-F0-2-r17 | Not checked |  |  |
| cqi-4-BitsSubbandNTN-SharedSpectrumChAccess-r17 | Not checked |  |  |
| mux-HARQ-ACK-DiffPriorities-r17 | Not checked |  |  |
| ta-BasedPDC-NTN-SharedSpectrumChAccess-r17 | Not checked |  |  |
| ack-NACK-FeedbackForMulticastWithDCI-Enabler-r17 | Not checked |  |  |
| maxNumberG-RNTI-r17 | Not checked |  |  |
| dynamicMulticastDCI-Format4-2-r17 | Not checked |  |  |
| maxModulationOrderForMulticast-r17 | Not checked |  |  |
| dynamicSlotRepetitionMulticastTN-NonSharedSpectrumChAccess-r17 | Not checked |  |  |
| dynamicSlotRepetitionMulticastNTN-SharedSpectrumChAccess-r17 | Not checked |  |  |
| nack-OnlyFeedbackForMulticastWithDCI-Enabler-r17 | Not checked |  |  |
| ack-NACK-FeedbackForSPS-MulticastWithDCI-Enabler-r17 | Not checked |  |  |
| maxNumberG-CS-RNTI-r17 | Not checked |  |  |
| re-LevelRateMatchingForMulticast-r17 | Not checked |  |  |
| pdsch-1024QAM-2MIMO-FR1-r17 | Not checked |  |  |
| prs-MeasurementWithoutMG-r17 | Not checked |  |  |
| maxNumber-NGSO-SatellitesPerCarrier-r17 | Not checked |  |  |
| prs-ProcessingCapabilityOutsideMGinPPW-r17 | Not checked |  |  |
| srs-SemiPersistent-PosResourcesRRC-Inactive-r17 | Not checked |  |  |
| channelBWs-DL-SCS-120kHz-FR2-2-r17 | Not checked |  |  |
| channelBWs-UL-SCS-120kHz-FR2-2-r17 | Not checked |  |  |
| dmrs-BundlingPUSCH-RepTypeA-r17 | Not checked |  |  |
| dmrs-BundlingPUSCH-RepTypeB-r17 | Not checked |  |  |
| dmrs-BundlingPUSCH-multiSlot-r17 | Not checked |  |  |
| dmrs-BundlingPUCCH-Rep-r17 | Not checked |  |  |
| interSlotFreqHopInterSlotBundlingPUSCH-r17 | Not checked |  |  |
| interSlotFreqHopPUCCH-r17 | Not checked |  |  |
| dmrs-BundlingRestart-r17 | Not checked |  |  |
| dmrs-BundlingNonBackToBackTX-r17 | Not checked |  |  |
| maxDynamicSlotRepetitionForSPS-Multicast-r17 | Not checked |  |  |
| nack-OnlyFeedbackForSPS-MulticastWithDCI-Enabler-r17 | Not checked |  |  |
| sps-MulticastDCI-Format4-2-r17 | Not checked |  |  |
| sps-MulticastMultiConfig-r17 | Not checked |  |  |
| priorityIndicatorInDCI-Multicast-r17 | Not checked |  |  |
| priorityIndicatorInDCI-SPS-Multicast-r17 | Not checked |  |  |
| twoHARQ-ACK-CodebookForUnicastAndMulticast-r17 | Not checked |  |  |
| multiPUCCH-HARQ-ACK-ForMulticastUnicast-r17 | Not checked |  |  |
| releaseSPS-MulticastWithCS-RNTI-r17 | Not checked |  |  |
| posUE-TA-AutoAdjustment-r18 | Not checked |  |  |
| posSRS-ValidityAreaRRC-InactiveInitialUL-BWP-r18 | Not checked |  |  |
| posSRS-ValidityAreaRRC-InactiveOutsideInitialUL-BWP-r18 | Not checked |  |  |
| dl-PRS-MeasurementWithRxFH-RRC-ConnectedForRedCap-r18 | Not checked |  |  |
| posSRS-TxFH-RRC-ConnectedForRedCap-r18 | Not checked |  |  |
| posSRS-TxFH-RRC-InactiveForRedCap-r18 | Not checked |  |  |
| posSRS-BWA-RRC-Inactive-r18 | Not checked |  |  |
| posJointTriggerBySingleDCI-RRC-Connected-r18 | Not checked |  |  |
| dl-PRS-MeasurementWithRxFH-RRC-InactiveforRedCap-r18 | Not checked |  |  |
| dl-PRS-MeasurementWithRxFH-RRC-IdleforRedCap-r18 | Not checked |  |  |
| spatialAdaptation-CSI-Feedback-r18 | Not checked |  |  |
| spatialAdaptation-CSI-FeedbackPUSCH-r18 | Not checked |  |  |
| spatialAdaptation-CSI-FeedbackAperiodic-r18 | Not checked |  |  |
| spatialAdaptation-CSI-FeedbackPUCCH-r18 | Not checked |  |  |
| powerAdaptation-CSI-Feedback-r18 | Not checked |  |  |
| powerAdaptation-CSI-FeedbackPUSCH-r18 | Not checked |  |  |
| powerAdaptation-CSI-FeedbackAperiodic-r18 | Not checked |  |  |
| powerAdaptation-CSI-FeedbackPUCCH-r18 | Not checked |  |  |
| nes-CellDTX-DRX-r18 | Checked |  | pc_nes_CellDTX_r18 and/or pc_nes_CellDRX_r18 |
| nes-CellDTX-DRX-DCI2-9-r18 | Checked |  | pc_nes_CellDTX_DRX_DCI2_9_r18 |
| mixCodeBookSpatialAdaptation-r18 | Not checked |  |  |
| simultaneousCSI-SubReportsPerCC-r18 | Not checked |  |  |
| ntn-DMRS-BundlingNGSO-r18 | Not checked |  |  |
| ltm-BeamIndicationJointTCI-r18 | Checked |  | pc_ltm_BeamIndicationJointTCI_r18 |
| dummy-ltm-MAC-CE-JointTCI-r18 | Not checked |  |  |
| ltm-BeamIndicationSeparateTCI-r18 | Not checked |  |  |
| dummy-ltm-MAC-CE-SeparateTCI-r18 | Not checked |  |  |
| rach-EarlyTA-Measurement-r18 | Not checked |  |  |
| ue-TA-Measurement-r18 | Checked |  | pc_ue_TA_Measurement_r18 |
| ta-IndicationCellSwitch-r18 | Checked |  | pc_ta_IndicationCellSwitch_r18 |
| triggeredHARQ-CodebookRetxDCI-1-3-r18 | Not checked |  |  |
| unifiedJointTCI-MultiMAC-CE-DCI-1-3-r18 | Not checked |  |  |
| unifiedSeparateTCI-MultiMAC-CE-IntraCell-r18 | Not checked |  |  |
| multiPUSCH-CG-r18 | Not checked |  |  |
| multiPUSCH-ActiveConfiguredGrant-r18 | Not checked |  |  |
| jointReleaseDCI-r18 | Not checked |  |  |
| cg-PUSCH-UTO-UCI-Ind-r18 | Not checked |  |  |
| pdcch-MonitoringResumptionAfterUL-NACK-r18 | Not checked |  |  |
| support3MHz-ChannelBW-Symmetric-r18 | Not checked |  |  |
| support3MHz-ChannelBW-Asymmetric-r18 | Not checked |  |  |
| support12PRB-CORESET0-r18 | Not checked |  |  |
| nr-PDCCH-OverlapLTE-CRS-RE-r18 | Not checked |  |  |
| nr-PDCCH-OverlapLTE-CRS-RE-MultiPatterns-r18 | Not checked |  |  |
| nr-PDCCH-OverlapLTE-CRS-RE-Span-3-4-r18 | Not checked |  |  |
| twoRateMatchingEUTRA-CRS-patterns-3-4-r18 | Not checked |  |  |
| overlapRateMatchingEUTRA-CRS-Patterns-3-4-Diff-CS-Pool-r18 | Not checked |  |  |
| ncd-SSB-BWP-Wor-r18 | Not checked |  |  |
| rlm-BM-BFD-CSI-RS-OutsideActiveBWP-r18 | Not checked |  |  |
| prach-CoverageEnh-r18 | Checked |  | pc_prach_CoverageEnh_r18 |
| prach-Repetition-r18 | Not checked |  |  |
| dynamicWaveformSwitch-r18 | Not checked |  |  |
| dynamicWaveformSwitchPHR-r18 | Checked |  | pc_dynamicWaveformSwitchPHR_r18 |
| dynamicWaveformSwitchIntraCA-r18 | Not checked |  |  |
| multiPUSCH-SingleDCI-NonConsSlots-r18 | Not checked |  |  |
| pdc-maxNumberPRS-ResourceProcessedPerSlot-r18 | Not checked |  |  |
| intraSlot-PDSCH-MulticastInactive-r18 | Not checked |  |  |
| multicastInactive-r18 | Not checked |  |  |
| thresholdBasedMulticastResume-r18 | Not checked |  |  |
| lowerMSD-r18 | Not checked |  |  |
| lowerMSD-ENDC-r18 | Not checked |  |  |
| enhancedChannelRaster-r18 | Not checked |  |  |
| fastBeamSweepingMultiRx-r18 | Not checked |  |  |
| beamSweepingFactorReduction-r18 | Not checked |  |  |
| simultaneousReceptionTwoQCL-r18 | Not checked |  |  |
| measEnhCAInterFreqFR2-r18 | Not checked |  |  |
| tci-StateSwitchInd-r18 | Not checked |  |  |
| antennaArrayType-r18 | Not checked |  |  |
| locationBasedCondHandoverATG-r18 | eeeeChecked |  | pc_locationBasedCondHandoverATG_r18 |
| maxOutputPowerATG-r18 | Not checked |  |  |
| ltm-FastProcessingConfig-r18 | Not checked |  |  |
| measValidationReportEMR-r18 | Not checked |  |  |
| measValidationReportReselectionMeasurements-r18 | Not checked |  |  |
| eventA4BasedCondHandoverNES-r18 | Not checked |  |  |
| nesBasedCondHandoverWithDCI-r18 | Not checked |  |  |
| rach-LessHandoverCG-r18 | Not checked |  |  |
| rach-LessHandoverDG-r18 | Not checked |  |  |
| locationBasedCondHandoverEMC-r18 | Not checked |  |  |
| mt-CG-SDT-r18 | Checked |  | pc_mt_CG_SDT_r18 |
| posSRS-PreconfigureRRC-InactiveInitialUL-BWP-r18 | Not checked |  |  |
| posSRS-PreconfigureRRC-InactiveOutsideInitialUL-BWP-r18 | Not checked |  |  |
| cg-SDT-PeriodicityExt-r18 | Checked |  | pc_cg_SDT_PeriodicityExt_r18 |
| supportOf2RxXR-r18 | Checked |  | pc_supportOf2RxXR_r18 |
| condHandoverWithCandSCG-change-r18 | Checked |  | pc_condHandoverWithCandSCG_change_r18 |
| mac-ParametersPerBand-r18 | Not checked |  |  |
| channelBW-DL-NCR-r18 | Not checked |  |  |
| channelBW-UL-NCR-r18 | Not checked |  |  |
| ncr-PDSCH-64QAM-FR2-r18 | Not checked |  |  |
| ltm-MCG-IntraFreq-r18 | Checked |  | pc_ltm_MCG_IntraFreq_r18 |
| ltm-SCG-IntraFreq-r18 | Checked |  | pc_ltm_SCG_IntraFreq_r18 |
| ltm-MAC-CE-JointTCI-r18 | Checked |  | pc_ltm_MAC_CE_JointTCI_r18 |
| ltm-MAC-CE-SeparateTCI-r18 | Checked |  | pc_ltm_MAC_CE_SeparateTCI_r18 |
| } |  |  |  |
| } |  |  |  |
| supportedBandCombinationList SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination { |  |  |  |
| BandCombination SEQUENCE { |  |  |  |
| bandList SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters { |  |  |  |
| BandParameters CHOICE { |  |  |  |
| eutra SEQUENCE { |  |  |  |
| bandEUTRA | Not checked | FreqBandIndicatorEUTRA |  |
| ca-BandwidthClassDL-EUTRA | Not checked | CA-BandwidthClassEUTRA |  |
| ca-BandwidthClassUL-EUTRA | Not checked | CA-BandwidthClassEUTRA |  |
| } |  |  |  |
| nr SEQUENCE { |  |  |  |
| bandNR | Not checked | FreqBandIndicatorNR |  |
| ca-BandwidthClassDL-NR | Not checked | CA-BandwidthClassNR |  |
| ca-BandwidthClassUL-NR | Not checked | CA-BandwidthClassNR |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| featureSetCombination | Not checked | FeatureSetCombinationId |  |
| ca-ParametersEUTRA | Not checked | CA-ParametersEUTRA |  |
| ca-ParametersNR | Not checked | CA-ParametersNR |  |
| mrdc-Parameters | Not checked | MRDC-Parameters |  |
| supportedBandwidthCombinationSet | Not checked |  |  |
| powerClass-v1530 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| appliedFreqBandListFilter | Not checked | FreqBandList |  |
| supportedBandCombinationList-v1610 SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1610 { |  | BandCombinationList-v1610 |  |
| BandCombination-v1610 SEQUENCE { |  |  |  |
| bandList-v1610 SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters-v1610 { |  |  |  |
| ca-ParametersNR-v1610 | Checked | CA-ParametersNR-v1610 (Table 8.1.5.1.1.3.3-8 |  |
| ca-ParametersNRDC-v1610 | Not checked | CA-ParametersNRDC-v1610 |  |
| powerClass-v1610 | Not checked |  |  |
| powerClassNRPart-r16 | Not checked |  |  |
| featureSetCombinationDAPS-r16 | Not checked | FeatureSetCombinationId |  |
| mrdc-Parameters-v1620 | Not checked | MRDC-Parameters-v1620 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| supportedBandCombinationListSidelinkEUTRA-NR-r16 | Not checked | BandCombinationListSidelinkEUTRA-NR-r16 |  |
| supportedBandCombinationList-UplinkTxSwitch-r16 | Not checked | BandCombinationList-UplinkTxSwitch-r16 |  |
| extendedBand-n77-r16 | Checked |  | pc_extendedBand_n77_r16 |
| supportedBandCombinationList-v1700 | Not checked | BandCombinationList-v1700 |  |
| supportedBandCombinationList-UplinkTxSwitch-v1700 | Not checked | BandCombinationList-UplinkTxSwitch-v1700 |  |
| supportedBandCombinationListSL-RelayDiscovery-r17 | Not checked |  |  |
| supportedBandCombinationListSL-NonRelayDiscovery-r17 | Not checked |  |  |
| supportedBandCombinationListSidelinkEUTRA-NR-v1710 | Not checked | BandCombinationListSidelinkEUTRA-NR-v1710 |  |
| sidelinkRequested-r17 | Not checked |  |  |
| extendedBand-n77-2-r17 | Checked |  | pc_extendedBand_n77_2_r17 |
| supportedBandCombinationList-v1800 SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1800 { |  | BandCombinationList-v1800 |  |
| BandCombination-v1800 SEQUENCE { |  |  |  |
| ca-ParametersNR-v1800 | Checked | CA-ParametersNR-v1800 (Table 8.1.5.1.1.3.3-14) |  |
| ca-ParametersNRDC-v1800 | Not checked |  |  |
| supportedBandCombListPerBC-SL-U2U-RelayDiscovery-r18 | Not checked |  |  |
| bandList-v1810 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| supportedBandCombinationList-UplinkTxSwitch-v1800 | Not checked |  |  |
| supportedBandCombinationListSL-U2U-Relay-r18 | Not checked |  |  |
| } |  |  |  |
| measAndMobParameters SEQUENCE { |  |  |  |
| measAndMobParametersCommon SEQUENCE { |  |  |  |
| supportedGapPattern | Checked | Acc. to TS 38.306 [23] clause 4.2.9: Indicates measurement gap pattern(s) optionally supported by the UE for NR SA. The leading / leftmost bit (bit 0) corresponds to the gap pattern 2, the next bit corresponds to the gap pattern 3, as specified in TS 38.133 [10] and so on. The UE shall set the bits corresponding to the measurement gap pattern 13, 14, 17, 18 and 19 to 1 if the UE is an NR standalone capable UE that supports a band in FR2. | pc_gp2_nr, pc_gp3_nr, pc_gp4_nr, pc_gp5_nr, pc_gp6_nr, pc_gp7_nr, pc_gp8_nr, pc_gp9_nr, pc_gp10_nr, pc_gp11_nr, pc_gp12_nr, pc_gp15_nr, pc_gp16_nr, pc_gp17_nr, pc_gp18_nr, pc_gp19_nr, pc_gp20_nr, pc_gp21_nr, pc_gp22_nr, pc_gp23_nr |
| ssb-RLM | supported |  |  |
| ssb-AndCSI-RS-RLM | Not checked |  |  |
| eventB-MeasAndReport | Checked |  | pc_eventB_MeasAndReport |
| handoverFDD-TDD | Not checked |  |  |
| eutra-CGI-Reporting | Checked |  | pc_eutra_CGI_Reporting |
| nr-CGI-Reporting | Checked |  | pc_nr_CGI_Reporting |
| independentGapConfig | Checked |  | pc_independentGapConfig |
| periodicEUTRA-MeasAndReport | Checked |  | pc_periodicEUTRA_MeasAndReport |
| handoverFR1-FR2 | Not checked |  |  |
| maxNumberCSI-RS-RRM-RS-SINR | Not checked |  |  |
| reportAddNeighMeasForPeriodic-r16 | Not checked |  |  |
| condHandoverParametersCommon-r16 | Not checked |  |  |
| nr-NeedForGap-Reporting-r16 | Checked |  | pc_nr_NeedForGap_Reporting_r16 |
| supportedGapPattern-NRonly-r16 | Not checked |  |  |
| supportedGapPattern-NRonly-NEDC-r16 | Not checked |  |  |
| maxNumberCLI-RSSI-r16 | Not checked |  |  |
| maxNumberCLI-SRS-RSRP-r16 | Not checked |  |  |
| maxNumberPerSlotCLI-SRS-RSRP-r16 | Not checked |  |  |
| mfbi-IAB-r16 | Not checked |  |  |
| dummy | Not checked |  |  |
| nr-CGI-Reporting-NPN-r16 | Checked |  | pc_nr_CGI_Reporting_NPN_r16 |
| idleInactiveEUTRA-MeasReport-r16 | Checked |  | pc_idleInactiveEUTRA_MeasReport |
| idleInactive-ValidityArea-r16 | Not checked |  |  |
| eutra-AutonomousGaps-r16 | Not checked |  |  |
| eutra-AutonomousGaps-NEDC-r16 | Not checked |  |  |
| eutra-AutonomousGaps-NRDC-r16 | Not checked |  |  |
| pcellT312-r16 | Not checked |  |  |
| supportedGapPattern-r16 | Not checked |  |  |
| concurrentMeasGap-r17 | Not checked |  |  |
| nr-NeedForGapNCSG-Reporting-r17 | Not checked |  |  |
| eutra-NeedForGapNCSG-Reporting-r17 | Not checked |  |  |
| ncsg-MeasGapPerFR-r17 | Not checked |  |  |
| ncsg-MeasGapPatterns-r17 | Not checked |  |  |
| ncsg-MeasGapNR-Patterns-r17 | Not checked |  |  |
| preconfiguredUE-AutonomousMeasGap-r17 | Not checked |  |  |
| preconfiguredNW-ControlledMeasGap-r17 | Not checked |  |  |
| handoverFR1-FR2-2-r17 | Not checked |  |  |
| handoverFR2-1-FR2-2-r17 | Not checked |  |  |
| independentGapConfigPRS-r17 | Not checked |  |  |
| rrm-RelaxationRRC-ConnectedRedCap-r17 | Checked |  | pc_rrm_RelaxationRRC_ConnectedRedCap_r17 |
| parallelMeasurementGap-r17 | Not checked |  |  |
| condHandoverWithSCG-NRDC-r17 | Not checked |  |  |
| gNB-ID-LengthReporting-r17 | Not checked |  |  |
| gNB-ID-LengthReporting-ENDC-r17 | Not checked |  |  |
| gNB-ID-LengthReporting-NEDC-r17 | Not checked |  |  |
| gNB-ID-LengthReporting-NRDC-r17 | Not checked |  |  |
| gNB-ID-LengthReporting-NPN-r17 | Not checked |  |  |
| eventD1-MeasReportTrigger-r17 | Checked |  | pc_eventD1_MeasReportTrigger_r17 |
| independentGapConfig-maxCC-r17 | Not checked |  |  |
| l3-MeasUnknownSCellActivation-r18 | Not checked |  |  |
| shortMeasInterval-r18 | Not checked |  |  |
| nr-NeedForInterruptionReport-r18 | Checked |  | pc_nr_NeedForInterruptionReport_r18 |
| measSequenceConfig-r18 | Not checked |  |  |
| cellIndividualOffsetPerMeasEvent-r18 | Not checked |  |  |
| eventD2-MeasReportTrigger-r18 | Not checked |  |  |
| concurrentMeasGapsPreMG-r18 | Checked |  | pc_concurrentMeasGapsPreMG_r18 |
| dynamicCollision-r18 | Checked |  | pc_dynamicCollision_r18 |
| concurrentMeasGapsNCSG-r18 | Checked |  | pc_concurrentMeasGapsNCSG_r18 |
| eutra-NoGapMeasurementOutsideBWP-r18 | Checked |  | pc_eutra_NoGapMeasurementOutsideBWP_r18 |
| eutra-NoGapMeasurementInsideBWP-r18 | Checked |  | pc_eutra_NoGapMeasurementInsideBWP_r18 |
| eutra-MeasEMW-r18 | Checked |  | pc_eutra_MeasEMW_r18 |
| concurrentMeasCRS-InsideBWP-EUTRA-r18 | Checked |  | pc_concurrentMeasCRS_InsideBWP_EUTRA_r18 |
| ltm-InterFreqMeasGap-r18 | Not checked |  |  |
| dummy-ltm-FastUE-Processing-r18 | Not checked |  |  |
| rach-LessHandoverInterFreq-r18 | Not checked |  |  |
| enterAndLeaveCellReport-r18 | Not checked |  |  |
| bestCellChangeReport-r18 | Not checked |  |  |
| secondBestCellChangeReport-r18 | Not checked |  |  |
| ltm-InterFreq-r18 | Checked |  | pc_ltm_InterFreq_r18 |
| ltm-MCG-NRDC-r18 | Checked |  | pc_ltm_MCG_NRDC_r18 |
| ltm-RACH-LessDG-r18 | Checked |  | pc_ltm_RACH_LessDG_r18 |
| ltm-RACH-LessCG-r18 | Checked |  | pc_ltm_RACH_LessCG_r18 |
| ltm-Recovery-r18 | Checked |  | pc_ltm_Recovery_r18 |
| ltm-ReferenceConfig-r18 | Checked |  | pc_ltm_ReferenceConfig_r18 |
| ltm-MCG-NRDC-Release-r18 | Checked |  | pc_ltm_MCG_NRDC_Release_r18 |
| ltm-FastUE-Processing-r18 SEQUENCE { |  |  |  |
| fr1-r18 | Checked |  | pc_ltm_FastUE_Processing_fr1_r18 |
| fr2-r18 | Checked |  | pc_ltm_FastUE_Processing_fr2_r18 |
| fr1-AndFR2-r18 | Checked |  | pc_ltm_FastUE_Processing_fr1_AndFR2_r18 |
| } |  |  |  |
| ntn-NeighbourCellInfoSupport-r18 | Not checked |  |  |
| ltm-interFreqL1-OnlyInBC-r18 | Checked |  | pc_ltm_interFreqL1_OnlyInBC_r18 |
| } |  |  |  |
| measAndMobParametersXDD-Diff SEQUENCE { |  |  |  |
| intraAndInterF-MeasAndReport | Checked (NOTE 1) |  | pc_intraAndInterF_MeasAndReport |
| eventA-MeasAndReport | Checked (NOTE 2) |  | pc_eventA_MeasAndReport |
| handoverInterF | Checked (NOTE 15) |  | pc_handoverInterF |
| handoverLTE-EPC | Not checked |  |  |
| handoverLTE-5GC | Not checked |  |  |
| sftd-MeasNR-Neigh | Checked (NOTE 13) |  | pc_SFTD_MeasNR_Neigh_FDD and/or pc_SFTD_MeasNR_Neigh_TDD |
| sftd-MeasNR-Neigh-DRX | Not checked |  |  |
| } |  |  |  |
| measAndMobParametersFRX-Diff SEQUENCE { |  |  |  |
| ss-SINR-Meas | Checked (NOTE 10) |  | pc_ss_SINR_Meas |
| csi-RSRP-AndRSRQ-MeasWithSSB | Checked (NOTE 3) |  | pc_csi_RSRP_AndRSRQ_MeasWithSSB |
| csi-RSRP-AndRSRQ-MeasWithoutSSB | Checked (NOTE 11) |  | pc_csi_RSRP_AndRSRQ_MeasWithoutSSB |
| csi-SINR-Meas | Not checked |  |  |
| csi-RS-RLM | Not checked |  |  |
| handoverInterF | Checked (NOTE 14) |  | pc_handoverInterF |
| handoverLTE-EPC | Not checked |  |  |
| handoverLTE-5GC | Not checked |  |  |
| nr-AutonomousGaps-r16 | Not checked |  |  |
| nr-AutonomousGaps-ENDC-r16 | Not checked |  |  |
| nr-AutonomousGaps-NEDC-r16 | Not checked |  |  |
| nr-AutonomousGaps-NRDC-r16 | Not checked |  |  |
| dummy | Not checked |  |  |
| cli-RSSI-Meas-r16 | Not checked |  |  |
| cli-SRS-RSRP-Meas-r16 | Not checked |  |  |
| interFrequencyMeas-NoGap-r16 | Not checked |  |  |
| simultaneousRxDataSSB-DiffNumerology-Inter-r16 | Not checked |  |  |
| idleInactiveNR-MeasReport-r16 | Checked (NOTE 22) |  | pc_idleInactiveNR_MeasReport |
| idleInactiveNR-MeasBeamReport-r16 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| fdd-Add-UE-NR-Capabilities SEQUENCE { |  |  |  |
| phy-ParametersXDD-Diff SEQUENCE { |  |  |  |
| dynamicSFI | Not checked |  |  |
| twoPUCCH-F0-2-ConsecSymbols | Not checked |  |  |
| twoDifferentTPC-Loop-PUSCH | Not checked |  |  |
| twoDifferentTPC-Loop-PUCCH | Not checked |  |  |
| dl-SchedulingOffset-PDSCH-TypeA | Checked (NOTE 17) |  | pc_dl_SchedulingOffset_PDSCH_TypeA |
| dl-SchedulingOffset-PDSCH-TypeB | Not checked |  |  |
| ul-SchedulingOffset | Not checked |  |  |
| } |  |  |  |
| mac-ParametersXDD-Diff SEQUENCE { |  |  |  |
| skipUplinkTxDynamic | Checked (NOTE 4) |  | pc_skipUplinkTxDynamic |
| logicalChannelSR-DelayTimer | Checked (NOTE 5) |  | pc_logicalChannelSR_DelayTimer |
| longDRX-Cycle | Checked (NOTE 6) |  | pc_longDRX_Cycle |
| shortDRX-Cycle | Checked (NOTE 7) |  | pc_shortDRX_Cycle |
| multipleSR-Configurations | Not checked |  |  |
| multipleConfiguredGrants | Checked (NOTE 12) |  | pc_multipleConfiguredGrants |
| } |  |  |  |
| measAndMobParametersXDD-Diff SEQUENCE { |  |  |  |
| intraAndInterF-MeasAndReport | Checked (NOTE 1) |  | pc_intraAndInterF_MeasAndReport |
| eventA-MeasAndReport | Checked (NOTE 2) |  | pc_eventA_MeasAndReport |
| handoverInterF | Checked (NOTE 15) |  | pc_handoverInterF |
| handoverLTE-EPC | Not checked |  |  |
| handoverLTE-5GC | Not checked |  |  |
| sftd-MeasNR-Neigh | Checked (NOTE 13) |  | pc_SFTD_MeasNR_Neigh_FDD |
| sftd-MeasNR-Neigh-DRX | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| tdd-Add-UE-NR-Capabilities SEQUENCE { |  |  |  |
| phy-ParametersXDD-Diff SEQUENCE { |  |  |  |
| dynamicSFI | Not checked |  |  |
| twoPUCCH-F0-2-ConsecSymbols | Not checked |  |  |
| twoDifferentTPC-Loop-PUSCH | Not checked |  |  |
| twoDifferentTPC-Loop-PUCCH | Not checked |  |  |
| dl-SchedulingOffset-PDSCH-TypeA | Checked (NOTE 17) |  | pc_dl_SchedulingOffset_PDSCH_TypeA |
| dl-SchedulingOffset-PDSCH-TypeB | Not checked |  |  |
| ul-SchedulingOffset | Not checked |  |  |
| } |  |  |  |
| mac-ParametersXDD-Diff SEQUENCE { |  |  |  |
| skipUplinkTxDynamic | Checked (NOTE 4) |  | pc_skipUplinkTxDynamic |
| logicalChannelSR-DelayTimer | Checked (NOTE 5) |  | pc_logicalChannelSR_DelayTimer |
| longDRX-Cycle | Checked (NOTE 6) |  | pc_longDRX_Cycle |
| shortDRX-Cycle | Checked (NOTE 7) |  | pc_shortDRX_Cycle |
| multipleSR-Configurations | Not checked |  |  |
| multipleConfiguredGrants | Checked (NOTE 12) |  | pc_multipleConfiguredGrants |
| } |  |  |  |
| measAndMobParametersXDD-Diff SEQUENCE { |  |  |  |
| intraAndInterF-MeasAndReport | Checked (NOTE 1) |  | pc_intraAndInterF_MeasAndReport |
| eventA-MeasAndReport | Checked (NOTE 2) |  | pc_eventA_MeasAndReport |
| handoverInterF | Checked (NOTE 15) |  | pc_handoverInterF |
| handoverLTE-EPC | Not checked |  |  |
| handoverLTE-5GC | Not checked |  |  |
| sftd-MeasNR-Neigh | Checked (NOTE 13) |  | pc_SFTD_MeasNR_Neigh_TDD |
| sftd-MeasNR-Neigh-DRX | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| fr1-Add-UE-NR-Capabilities SEQUENCE { |  |  |  |
| phy-ParametersFRX-Diff SEQUENCE { |  |  |  |
| dynamicSFI | Not checked |  |  |
| dummy1 | Not checked |  |  |
| twoFL-DMRS | Not checked |  |  |
| dummy2 | Not checked |  |  |
| dummy3 | Not checked |  |  |
| supportedDMRS-TypeDL | Not checked |  |  |
| supportedDMRS-TypeUL | Not checked |  |  |
| semiOpenLoopCSI | Not checked |  |  |
| csi-ReportWithoutPMI | Not checked |  |  |
| csi-ReportWithoutCQI | Not checked |  |  |
| onePortsPTRS | Not checked |  |  |
| twoPUCCH-F0-2-ConsecSymbols | Not checked |  |  |
| pucch-F2-WithFH | Not checked |  |  |
| pucch-F3-WithFH | Not checked |  |  |
| pucch-F4-WithFH | Not checked |  |  |
| pucch-F0-2WithoutFH | Not checked |  |  |
| pucch-F1-3-4WithoutFH | Not checked |  |  |
| mux-SR-HARQ-ACK-CSI-PUCCH-MultiPerSlot | Not checked |  |  |
| uci-CodeBlockSegmentation | Not checked |  |  |
| onePUCCH-LongAndShortFormat | Not checked |  |  |
| twoPUCCH-AnyOthersInSlot | Checked (NOTE 26) |  | pc_twoPUCCH_AnyOthersInSlot |
| intraSlotFreqHopping-PUSCH | Not checked |  |  |
| pusch-LBRM | Not checked |  |  |
| pdcch-BlindDetectionCA | Not checked |  |  |
| tpc-PUSCH-RNTI | Not checked |  |  |
| tpc-PUCCH-RNTI | Not checked |  |  |
| tpc-SRS-RNTI | Not checked |  |  |
| absoluteTPC-Command | Not checked |  |  |
| twoDifferentTPC-Loop-PUSCH | Not checked |  |  |
| twoDifferentTPC-Loop-PUCCH | Not checked |  |  |
| pusch-HalfPi-BPSK | Not checked |  |  |
| pucch-F3-4-HalfPi-BPSK | Not checked |  |  |
| almostContiguousCP-OFDM-UL | Not checked |  |  |
| sp-CSI-RS | Not checked |  |  |
| sp-CSI-IM | Not checked |  |  |
| tdd-MultiDL-UL-SwitchPerSlot | Not checked |  |  |
| multipleCORESET | Not checked |  |  |
| csi-RS-IM-ReceptionForFeedback | Not checked |  |  |
| csi-RS-ProcFrameworkForSRS | Not checked |  |  |
| csi-ReportFramework | Not checked |  |  |
| mux-SR-HARQ-ACK-CSI-PUCCH-OncePerSlot SEQUENCE { |  |  |  |
| sameSymbol | Not checked |  |  |
| diffSymbol | Not checked |  |  |
| } |  |  |  |
| mux-SR-HARQ-ACK-PUCCH | Not checked |  |  |
| mux-MultipleGroupCtrlCH-Overlap | Not checked |  |  |
| dl-SchedulingOffset-PDSCH-TypeA | Checked (NOTE 18) |  | pc_dl_SchedulingOffset_PDSCH_TypeA |
| dl-SchedulingOffset-PDSCH-TypeB | Not checked |  |  |
| ul-SchedulingOffset | Not checked |  |  |
| dl-64QAM-MCS-TableAlt | Not checked |  |  |
| ul-64QAM-MCS-TableAlt | Not checked |  |  |
| cqi-TableAlt | Not checked |  |  |
| oneFL-DMRS-TwoAdditionalDMRS-UL | Not checked |  |  |
| twoFL-DMRS-TwoAdditionalDMRS-UL | Not checked |  |  |
| oneFL-DMRS-ThreeAdditionalDMRS-UL | Not checked |  |  |
| } |  |  |  |
| measAndMobParametersFRX-Diff SEQUENCE { |  |  |  |
| ss-SINR-Meas | Checked (NOTE 10) |  | pc_ss_SINR_Meas |
| csi-RSRP-AndRSRQ-MeasWithSSB | Checked (NOTE 3) |  | pc_csi_RSRP_AndRSRQ_MeasWithSSB |
| csi-RSRP-AndRSRQ-MeasWithoutSSB | Checked (NOTE 11) |  | pc_csi_RSRP_AndRSRQ_MeasWithoutSSB |
| csi-SINR-Meas | Not checked |  |  |
| csi-RS-RLM | Not checked |  |  |
| handoverInterF | Checked (NOTE 14) |  | pc_handoverInterF |
| handoverLTE-EPC | Not checked |  |  |
| handoverLTE-5GC | Not checked |  |  |
| nr-AutonomousGaps-r16 | Not checked |  |  |
| nr-AutonomousGaps-ENDC-r16 | Not checked |  |  |
| nr-AutonomousGaps-NEDC-r16 | Not checked |  |  |
| nr-AutonomousGaps-NRDC-r16 | Not checked |  |  |
| dummy | Not checked |  |  |
| cli-RSSI-Meas-r16 | Not checked |  |  |
| cli-SRS-RSRP-Meas-r16 | Not checked |  |  |
| interFrequencyMeas-NoGap-r16 | Not checked |  |  |
| simultaneousRxDataSSB-DiffNumerology-Inter-r16 | Not checked |  |  |
| idleInactiveNR-MeasReport-r16 | Checked (NOTE 22) |  | pc_idleInactiveNR_MeasReport |
| idleInactiveNR-MeasBeamReport-r16 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| fr2-Add-UE-NR-Capabilities SEQUENCE { |  |  |  |
| phy-ParametersFRX-Diff SEQUENCE { | Not checked |  |  |
| dynamicSFI | Not checked |  |  |
| dummy1 | Not checked |  |  |
| twoFL-DMRS | Not checked |  |  |
| dummy2 | Not checked |  |  |
| dummy3 | Not checked |  |  |
| supportedDMRS-TypeDL | Not checked |  |  |
| supportedDMRS-TypeUL | Not checked |  |  |
| semiOpenLoopCSI | Not checked |  |  |
| csi-ReportWithoutPMI | Not checked |  |  |
| csi-ReportWithoutCQI | Not checked |  |  |
| onePortsPTRS | Not checked |  |  |
| twoPUCCH-F0-2-ConsecSymbols | Not checked |  |  |
| pucch-F2-WithFH | Not checked |  |  |
| pucch-F3-WithFH | Not checked |  |  |
| pucch-F4-WithFH | Not checked |  |  |
| pucch-F0-2WithoutFH | Not checked |  |  |
| pucch-F1-3-4WithoutFH | Not checked |  |  |
| mux-SR-HARQ-ACK-CSI-PUCCH-MultiPerSlot | Not checked |  |  |
| uci-CodeBlockSegmentation | Not checked |  |  |
| onePUCCH-LongAndShortFormat | Not checked |  |  |
| twoPUCCH-AnyOthersInSlot | Checked (NOTE 26) |  | pc_twoPUCCH_AnyOthersInSlot |
| intraSlotFreqHopping-PUSCH | Not checked |  |  |
| pusch-LBRM | Not checked |  |  |
| pdcch-BlindDetectionCA | Not checked |  |  |
| tpc-PUSCH-RNTI | Not checked |  |  |
| tpc-PUCCH-RNTI | Not checked |  |  |
| tpc-SRS-RNTI | Not checked |  |  |
| absoluteTPC-Command | Not checked |  |  |
| twoDifferentTPC-Loop-PUSCH | Not checked |  |  |
| twoDifferentTPC-Loop-PUCCH | Not checked |  |  |
| pusch-HalfPi-BPSK | Not checked |  |  |
| pucch-F3-4-HalfPi-BPSK | Not checked |  |  |
| almostContiguousCP-OFDM-UL | Not checked |  |  |
| sp-CSI-RS | Not checked |  |  |
| sp-CSI-IM | Not checked |  |  |
| tdd-MultiDL-UL-SwitchPerSlot | Not checked |  |  |
| multipleCORESET | Not checked |  |  |
| csi-RS-IM-ReceptionForFeedback | Not checked |  |  |
| csi-RS-ProcFrameworkForSRS | Not checked |  |  |
| csi-ReportFramework | Not checked |  |  |
| mux-SR-HARQ-ACK-CSI-PUCCH-OncePerSlot SEQUENCE { |  |  |  |
| sameSymbol | Not checked |  |  |
| diffSymbol | Not checked |  |  |
| } |  |  |  |
| mux-SR-HARQ-ACK-PUCCH | Not checked |  |  |
| mux-MultipleGroupCtrlCH-Overlap | Not checked |  |  |
| dl-SchedulingOffset-PDSCH-TypeA | Checked (NOTE 18) |  | pc_dl_SchedulingOffset_PDSCH_TypeA |
| dl-SchedulingOffset-PDSCH-TypeB | Not checked |  |  |
| ul-SchedulingOffset | Not checked |  |  |
| dl-64QAM-MCS-TableAlt | Not checked |  |  |
| ul-64QAM-MCS-TableAlt | Not checked |  |  |
| cqi-TableAlt | Not checked |  |  |
| oneFL-DMRS-TwoAdditionalDMRS-UL | Not checked |  |  |
| twoFL-DMRS-TwoAdditionalDMRS-UL | Not checked |  |  |
| oneFL-DMRS-ThreeAdditionalDMRS-UL | Not checked |  |  |
| } |  |  |  |
| measAndMobParametersFRX-Diff SEQUENCE { |  |  |  |
| ss-SINR-Meas | Checked (NOTE 10) |  | pc_ss_SINR_Meas |
| csi-RSRP-AndRSRQ-MeasWithSSB | Checked (NOTE 3) |  | pc_csi_RSRP_AndRSRQ_MeasWithSSB |
| csi-RSRP-AndRSRQ-MeasWithoutSSB | Checked (NOTE 11) |  | pc_csi_RSRP_AndRSRQ_MeasWithoutSSB |
| csi-SINR-Meas | Not checked |  |  |
| csi-RS-RLM | Not checked |  |  |
| handoverInterF | Checked (NOTE 14) |  | pc_handoverInterF |
| handoverLTE-EPC | Not checked |  |  |
| handoverLTE-5GC | Not checked |  |  |
| nr-AutonomousGaps-r16 | Not checked |  |  |
| nr-AutonomousGaps-ENDC-r16 | Not checked |  |  |
| nr-AutonomousGaps-NEDC-r16 | Not checked |  |  |
| nr-AutonomousGaps-NRDC-r16 | Not checked |  |  |
| dummy | Not checked |  |  |
| cli-RSSI-Meas-r16 | Not checked |  |  |
| cli-SRS-RSRP-Meas-r16 | Not checked |  |  |
| interFrequencyMeas-NoGap-r16 | Not checked |  |  |
| simultaneousRxDataSSB-DiffNumerology-Inter-r16 | Not checked |  |  |
| idleInactiveNR-MeasReport-r16 | Checked (NOTE 22) |  | pc_idleInactiveNR_MeasReport |
| idleInactiveNR-MeasBeamReport-r16 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| featureSets SEQUENCE { |  |  |  |
| featureSetsDownlink SEQUENCE (SIZE (1..maxDownlinkFeatureSets)) OF FeatureSetDownlink { |  |  |  |
| FeatureSetDownlink SEQUENCE { | Not checked |  |  |
| featureSetListPerDownlinkCC | Not checked |  |  |
| intraBandFreqSeparationDL | Not checked |  |  |
| scalingFactor | Not checked |  |  |
| dummy8 | Not checked |  |  |
| scellWithoutSSB | Not checked |  |  |
| csi-RS-MeasSCellWithoutSSB | Not checked |  |  |
| dummy1 | Not checked |  |  |
| type1-3-CSS | Not checked |  |  |
| pdcch-MonitoringAnyOccasions | Not checked |  |  |
| dummy2 | Not checked |  |  |
| ue-SpecificUL-DL-Assignment | Not checked |  |  |
| searchSpaceSharingCA-DL | Not checked |  |  |
| timeDurationForQCL SEQUENCE { |  |  |  |
| scs-60kHz | Not checked |  |  |
| scs-120kHz | Not checked |  |  |
| } |  |  |  |
| pdsch-ProcessingType1-DifferentTB-PerSlot SEQUENCE { |  |  |  |
| scs-15kHz | Not checked |  |  |
| scs-30kHz | Not checked |  |  |
| scs-60kHz | Not checked |  |  |
| scs-120kHz | Not checked |  |  |
| } |  |  |  |
| dummy3 | Not checked |  |  |
| dummy4 | Not checked |  |  |
| dummy5 | Not checked |  |  |
| dummy6 | Not checked |  |  |
| dummy7 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| featureSetsDownlinkPerCC SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetDownlinkPerCC { |  |  |  |
| FeatureSetDownlinkPerCC SEQUENCE { |  |  |  |
| supportedSubcarreirSpacingDL | Not checked | SubcarrierSpacing |  |
| supportedBandwidthDL | Not checked | SupportedBandwidth |  |
| channelBW-90mhz | Not checked |  |  |
| maxNumberMIMO-LayersPDSCH | Checked | MIMO-LayersDL | pc_maxNumberMIMO_LayersPDSCH_eightLayers or pc_maxNumberMIMO_LayersPDSCH_fourLayers or pc_maxNumberMIMO_LayersPDSCH_twoLayers |
| supportedModulationOrderDL | Not checked | ModulationOrder |  |
| } |  |  |  |
| } |  |  |  |
| featureSetsUplink SEQUENCE (SIZE (1..maxUplinkFeatureSets)) OF FeatureSetUplink { |  |  |  |
| FeatureSetUplink SEQUENCE { |  |  |  |
| featureSetListPerUplinkCC | Not checked |  |  |
| scalingFactor | Not checked |  |  |
| dummy3 | Not checked |  |  |
| intraBandFreqSeparationUL | Not checked |  |  |
| searchSpaceSharingCA-UL | Not checked |  |  |
| dummy1 | Not checked |  |  |
| supportedSRS-Resources | Not checked | SRS-Resources |  |
| twoPUCCH-Group | Checked |  | pc_twoPUCCH_group |
| dynamicSwitchSUL | Checked |  | pc_dynamicSwitch_SUL |
| simultaneousTxSUL-NonSUL | Not checked |  |  |
| pusch-ProcessingType1-DifferentTB-PerSlot SEQUENCE { |  |  |  |
| scs-15kHz | Not checked |  |  |
| scs-30kHz | Not checked |  |  |
| scs-60kHz | Not checked |  |  |
| scs-120kHz | Not checked |  |  |
| } |  |  |  |
| dummy2 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| featureSetsUplinkPerCC SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetUplinkPerCC { |  |  |  |
| FeatureSetUplinkPerCC SEQUENCE { |  |  |  |
| supportedSubcarrierSpacingUL | Not checked | SubcarrierSpacing |  |
| supportedBandwidthUL | Not checked | SupportedBandwidth |  |
| channelBW-90mHz | Not checked |  |  |
| mimo-CB-PUSCH SEQUENCE { | Checked |  | pc_nrMIMO_CB_PUSCH |
| maxNumberMIMO-LayersCB-PUSCH | Checked |  | pc_maxNumberMIMO_LayersCB_PUSCH_fourlayer or pc_maxNumberMIMO_LayersCB_PUSCH_twolayer |
| maxNumberSRS-ResourcePerSet | Not checked |  |  |
| } |  |  |  |
| maxNumberMIMO-LayersNonCB-PUSCH | Not checked |  |  |
| supportedModulationOrderUL | Not checked | ModulationOrder |  |
| } |  |  |  |
| } |  |  |  |
| featureSetsDownlink-v1610 SEQUENCE (SIZE (1.. maxDownlinkFeatureSets)) OF FeatureSetDownlink-v1610 | Checked | FeatureSetDownlink-v1610 (Table 8.1.5.1.1.3.3-6) |  |
| featureSetsUplink-v1610 SEQUENCE (SIZE (1.. maxUplinkFeatureSets)) OF FeatureSetUplink-v1610 | Checked | FeatureSetUplink-v1610 (Table 8.1.5.1.1.3.3-7) |  |
| featureSetDownlinkPerCC-v1620 SEQUENCE (SIZE (1.. maxPerCC-FeatureSets)) OF FeatureSetDownlinkPerCC-v1620 | Checked | FeatureSetDownlinkPerCC-v1620 (Table 8.1.5.1.1.3.3-7a) |  |
| featureSetsDownlink-v1700 | Not checked |  |  |
| featureSetsDownlinkPerCC-v1700 SEQUENCE (SIZE (1..maxPerCC-FeatureSets)) OF FeatureSetDownlinkPerCC-v1700 | Checked | FeatureSetDownlinkPerCC- v1700 (Table 8.1.5.1.1.3.3-7d) |  |
| featureSetsUplink-v1710 SEQUENCE (SIZE (1.. maxUplinkFeatureSets)) OF FeatureSetUplink-v1710 | Checked | FeatureSetUplink-v1710 (Table 8.1.5.1.1.3.3-7b) |  |
| featureSetsUplinkPerCC-v1700 SEQUENCE (SIZE (1.. maxPerCC-FeatureSets)) OF FeatureSetUplinkPerCC-v1700 | Checked | FeatureSetUplinkPerCC-v1700 (Table 8.1.5.1.1.3.3-7c) |  |
| } |  |  |  |
| featureSetCombinations SEQUENCE (SIZE (1..maxFeatureSetCombinations)) OF FeatureSetCombination { |  |  |  |
| FeatureSetCombination SEQUENCE (SIZE (1..maxSimultaneousBands)) OF FeatureSetsPerBand { |  |  |  |
| FeatureSetsPerBand SEQUENCE (SIZE (1..maxFeatureSetsPerBand)) OF FeatureSet { |  |  |  |
| FeatureSetCHOICE { |  |  |  |
| eutra SEQUENCE { |  |  |  |
| downlinkSetEUTRA | Not checked | FeatureSetEUTRA-DownlinkId |  |
| uplinkSetEUTRA | Not checked | FeatureSetEUTRA-UplinkId |  |
| } |  |  |  |
| nr SEQUENCE { |  |  |  |
| downlinkSetNR | Not checked | FeatureSetDownlinkId |  |
| uplinkSetNR | Not checked | FeatureSetUpinkId |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| fdd-Add-UE-NR-Capabilities-v1530 SEQUENCE { |  |  |  |
| eutra-ParametersXDD-Diff SEQUENCE { |  |  |  |
| rsrqMeasWidebandEUTRA | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| tdd-Add-UE-NR-Capabilities-v1530 SEQUENCE { |  |  |  |
| eutra-ParametersXDD-Diff SEQUENCE { |  |  |  |
| rsrqMeasWidebandEUTRA | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| dummy | Not checked |  |  |
| interRAT-Parameters SEQUENCE { |  |  |  |
| eutra SEQUENCE { |  |  |  |
| supportedBandListEUTRA SEQUENCE (SIZE (1..maxBandsEUTRA)) OF FreqBandIndicatorEUTRA | Not checked |  |  |
| eutra-ParametersCommon SEQUENCE { |  |  |  |
| mfbi-EUTRA | Not checked |  |  |
| modifiedMRP-BehaviorEUTRA | Not checked |  |  |
| multiNS-Pmax-EUTRA | Not checked |  |  |
| rs-SINR-MeasEUTRA | Checked |  | pc_RS_SINR_MeasEUTRA |
| ne-DC | Not checked |  |  |
|  | Not Present |  | pc_supportOfRedCap_r17 |
| nr-HO-ToEN-DC-r16 | Checked |  | pc_interRAT_NR_ToENDC |
|  | Not Present |  | pc_supportOfRedCap_r17 |
| } |  |  |  |
| eutra-ParametersXDD-Diff SEQUENCE { |  |  |  |
| rsrqMeasWidebandEUTRA | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| inactiveState | Checked |  | pc_inactiveState |
| delayBudgetReporting | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  | UE-NR-Capability-v1540 |  |
| sdap-Parameters SEQUENCE { |  |  |  |
| as-ReflectiveQoS | Checked |  | pc_as_ReflectiveQoS |
| } |  |  |  |
| overheatingInd | Not checked |  |  |
| ims-Parameters SEQUENCE { |  |  |  |
| ims-ParametersCommon SEQUENCE { |  |  |  |
| voiceOverEUTRA-5GC | Not checked |  |  |
| voiceOverSCG-BearerEUTRA-5GC | Not checked |  |  |
| voiceFallbackIndicationEPS-r16 | Checked |  | pc_voiceFallbackIndication |
| } |  |  |  |
| ims-ParametersFRX-Diff SEQUENCE { |  |  |  |
| voiceOverNR | Checked (NOTE 16) |  | pc_voiceOverNR |
| } |  |  |  |
| } |  |  |  |
| fr1-Add-UE-NR-Capabilities-v1540 SEQUENCE { |  |  |  |
| ims-ParametersFRX-Diff SEQUENCE { |  |  |  |
| voiceOverNR | Checked (NOTE 16) |  | pc_voiceOverNR |
| } |  |  |  |
| } |  |  |  |
| fr2-Add-UE-NR-Capabilities-v1540 SEQUENCE { |  |  |  |
| ims-ParametersFRX-Diff SEQUENCE { |  |  |  |
| voiceOverNR | Checked (NOTE 16) |  | pc_voiceOverNR |
| } |  |  |  |
| } |  |  |  |
| fr1-fr2-Add-UE-NR-Capabilities SEQUENCE { |  |  |  |
| phy-ParametersFRX-Diff SEQUENCE { |  |  |  |
| dynamicSFI | Not checked |  |  |
| dummy1 | Not checked |  |  |
| twoFL-DMRS | Not checked |  |  |
| dummy2 | Not checked |  |  |
| dummy3 | Not checked |  |  |
| supportedDMRS-TypeDL | Not checked |  |  |
| supportedDMRS-TypeUL | Not checked |  |  |
| semiOpenLoopCSI | Not checked |  |  |
| csi-ReportWithoutPMI | Not checked |  |  |
| csi-ReportWithoutCQI | Not checked |  |  |
| onePortsPTRS | Not checked |  |  |
| twoPUCCH-F0-2-ConsecSymbols | Not checked |  |  |
| pucch-F2-WithFH | Not checked |  |  |
| pucch-F3-WithFH | Not checked |  |  |
| pucch-F4-WithFH | Not checked |  |  |
| pucch-F0-2WithoutFH | Not checked |  |  |
| pucch-F1-3-4WithoutFH | Not checked |  |  |
| mux-SR-HARQ-ACK-CSI-PUCCH-MultiPerSlot | Not checked |  |  |
| uci-CodeBlockSegmentation | Not checked |  |  |
| onePUCCH-LongAndShortFormat | Not checked |  |  |
| twoPUCCH-AnyOthersInSlot | Not checked |  |  |
| intraSlotFreqHopping-PUSCH | Not checked |  |  |
| pusch-LBRM | Not checked |  |  |
| pdcch-BlindDetectionCA | Not checked |  |  |
| tpc-PUSCH-RNTI | Not checked |  |  |
| tpc-PUCCH-RNTI | Not checked |  |  |
| tpc-SRS-RNTI | Not checked |  |  |
| absoluteTPC-Command | Not checked |  |  |
| twoDifferentTPC-Loop-PUSCH | Not checked |  |  |
| twoDifferentTPC-Loop-PUCCH | Not checked |  |  |
| pusch-HalfPi-BPSK | Not checked |  |  |
| pucch-F3-4-HalfPi-BPSK | Not checked |  |  |
| almostContiguousCP-OFDM-UL | Not checked |  |  |
| sp-CSI-RS | Not checked |  |  |
| sp-CSI-IM | Not checked |  |  |
| tdd-MultiDL-UL-SwitchPerSlot | Not checked |  |  |
| multipleCORESET | Not checked |  |  |
| } |  |  |  |
| measAndMobParametersFRX-Diff SEQUENCE { |  |  |  |
| ss-SINR-Meas | Not checked |  |  |
| csi-RSRP-AndRSRQ-MeasWithSSB | Not checked |  |  |
| csi-RSRP-AndRSRQ-MeasWithoutSSB | Not checked |  |  |
| csi-SINR-Meas | Not checked |  |  |
| csi-RS-RLM | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  | UE-NR-Capability-v1550 |  |
| reducedCP-Latency | Checked |  | pc_reducedCP_Latency |
| nonCriticalExtension SEQUENCE { |  | UE-NR-Capability-v1560 |  |
| nrdc-Parameters | Checked | NRDC-Parameters (Table 8.1.5.1.1.3.3-15) |  |
| receivedFilters | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  | UE-NR-Capability-v1570 |  |
| nrdc-Parameters-v1570 | Not checked | NRDC-Parameters-v1570 |  |
| nonCriticalExtension SEQUENCE { |  | UE-NR-Capability-v1610 |  |
| inDeviceCoexInd-r16 | Checked |  | pc_inDeviceCoexInd_r16 |
| dl-DedicatedMessageSegmentation-r16 | Checked |  | pc_NR_dl_DedicatedMessageSegmentation |
| nrdc-Parameters-v1610 | Not checked |  |  |
| powSav-Parameters-r16 | Checked | PowSav-Parameters-r16 (Table 8.1.5.1.1.3.3-9) |  |
| fr1-Add-UE-NR-Capabilities-v1610 SEQUENCE { |  | UE-NR-CapabilityAddFRX-Mode-v1610 |  |
| powSav-ParametersFRX-Diff-r16 | Not checked |  |  |
| mac-ParametersFRX-Diff-r16 | Checked (NOTES 19, 23, 24, 25, 27) | MAC-ParametersFRX-Diff-r16 (Table 8.1.5.1.1.3.3-11) |  |
| } |  |  |  |
| fr2-Add-UE-NR-Capabilities-v1610 SEQUENCE { |  | UE-NR-CapabilityAddFRX-Mode-v1610 |  |
| powSav-ParametersFRX-Diff-r16 | Not checked |  |  |
| mac-ParametersFRX-Diff-r16 | Checked (NOTES 19, 23, 24, 25, 27) | MAC-ParametersFRX-Diff-r16 (Table 8.1.5.1.1.3.3-11) |  |
| } |  |  |  |
| bh-RLF-Indication-r16 | Not checked |  |  |
| directSN-AdditionFirstRRC-IAB-r16 | Not checked |  |  |
| bap-Parameters-r16 | Not checked |  |  |
| referenceTimeProvision-r16 | Not checked |  |  |
| sidelinkParameters-r16 SEQUENCE { |  |  |  |
| sidelinkParametersNR-r16 { |  |  |  |
| rlc-ParametersSidelink-r16 | Not checked |  |  |
| mac-ParametersSidelink-r16 | Not checked |  |  |
| fdd-Add-UE-Sidelink-Capabilities-r16 | Not checked |  |  |
| tdd-Add-UE-Sidelink-Capabilities-r16 | Not checked |  |  |
| supportedBandListSidelink-r16 SEQUENCE (SIZE (1..maxBands)) OF BandSidelink-r16 { |  |  |  |
| freqBandSidelink-r16 | Not checked |  |  |
| sl-Reception-r16 | Not checked |  |  |
| sl-TransmissionMode1-r16 | Checked |  | pc_NR_sl_TransmissionMode1_r16 |
| sync-Sidelink-r16 | Not checked |  |  |
| sl-Tx-256QAM-r16 | Not checked |  |  |
| psfch-FormatZeroSidelink-r16 | Not checked |  |  |
| lowSE-64QAM-MCS-TableSidelink-r16 | Not checked |  |  |
| enb-sync-Sidelink-r16 | Not checked |  |  |
| sl-TransmissionMode2-r16 | Checked | TS 38.306 [23] clause 4.2.16.1.6: Support of this feature is mandatory if UE supports NR sidelink | pc_NR_sl_TransmissionMode2_r16 |
| congestionControlSidelink-r16 | Not checked |  |  |
| fewerSymbolSlotSidelink-r16 | Not checked |  |  |
| sl-openLoopPC-RSRP-ReportSidelink-r16 | Not checked |  |  |
| sl-Rx-256QAM-r16 | Not checked |  |  |
| } |  |  |  |
| relayParameters-r17 | Not checked |  |  |
| p0_OLPC_Sidelink_r17 | Not checked |  |  |
| } |  |  |  |
| sidelinkParametersEUTRA-r16 | Not checked |  |  |
| } |  |  |  |
| highSpeedParameters-r16 | Not checked |  |  |
| mac-Parameters-v1610 SEQUENCE { |  | MAC-Parameters-v1610 |  |
| mac-ParametersFRX-Diff-r16 | Checked (NOTES 19, 23, 24, 25, 27) | MAC-ParametersFRX-Diff-r16 (Table 8.1.5.1.1.3.3-11) |  |
| } |  |  |  |
| mcgRLF-RecoveryViaSCG-r16 | Not checked |  |  |
| resumeWithStoredMCG-SCells-r16 | Not checked |  |  |
| resumeWithStoredSCG-r16 | Not checked |  |  |
| resumeWithSCG-Config-r16 | Checked |  | pc_resumeWithSCG_Config_r16 |
| ue-BasedPerfMeas-Parameters-r16 | Checked | UE-BasedPerfMeas-Parameters-r16 (Table 8.1.5.1.1.3.3-10) |  |
| son-Parameters-r16 SEQUENCE { |  |  |  |
| rach-Report-r16 | Checked |  | pc_rachReport_r16 |
| rlfReportCHO-r17 | Checked |  | pc_rlfReportCHO_r17 |
| rlfReportDAPS-r17 | Checked |  | pc_rlfReportDAPS_r17 |
| success-HO-Report-r17 | Checked |  | pc_success_HO_Report_r17 |
| twoStepRACH-Report-r17 | Checked |  | pc_twoStepRACH_Report_r17 |
| pscell-MHI-Report-r17 | Not checked |  |  |
| onDemandSI-Report-r17 | Checked |  | pc_onDemandSI_Report_r17 |
| cef-ReportRedCap-r17 | Checked |  | pc_cef_ReportRedCap_r17 |
| rlf-ReportRedCap-r17 | Checked |  | pc_rlf_ReportRedCap_r17 |
| } |  |  |  |
| onDemandSIB-Connected-r16 | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  | UE-NR-Capability-v1640 |  |
| redirectAtResumeByNAS-r16 | Not checked |  |  |
| phy-ParametersSharedSpectrumChAccess-r16 | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  | UE-NR-Capability-v1650 |  |
| mpsPriorityIndication-r16 | Checked |  | pc_NR_mpsPriorityIndication_r16 |
| highSpeedParameters-v1650 | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  | UE-NR-Capability-v1690 |  |
| ul-RRC-Segmentation-r16 | Checked |  | pc_NR_UL_Segmentation |
| nonCriticalExtension SEQUENCE { |  | UE-NR-Capability-v1700 |  |
| inactiveStatePO-Determination-r17 | Not checked |  |  |
| highSpeedParameters-v1700 | Not checked |  |  |
| powSav-Parameters-v1700 | Not checked |  |  |
| mac-Parameters-v1700 SEQUENCE { |  |  |  |
| mac-ParametersFR2-2-r17 SEQUENCE { |  | MAC-ParametersFR2-2-r17 |  |
| directMCG-SCellActivation-r17 | Not checked |  |  |
| directMCG-SCellActivationResume-r17 | Not checked |  |  |
| directSCG-SCellActivation-r17 | Not checked |  |  |
| directSCG-SCellActivationResume-r17 | Not checked |  |  |
| drx-Adaptation-r17 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| ims-Parameters-v1700 | Not checked |  |  |
| measAndMobParameters-v1700 | Not checked |  |  |
| appLayerMeasParameters-r17 SEQUENCE { |  |  |  |
| qoe-Streaming-MeasReport-r17 | Not checked |  |  |
| qoe-MTSI-MeasReport-r17 | Checked |  | pc_qoe_MTSI_MeasReport_r17 |
| qoe-VR-MeasReport-r17 | Not checked |  |  |
| ran-VisibleQoE-Streaming-MeasReport-r17 | Not checked |  |  |
| ran-VisibleQoE-VR-MeasReport-r17 | Not checked |  |  |
| ul-MeasurementReportAppLayer-Seg-r17 | Not checked |  |  |
| } |  |  |  |
| redCapParameters-r17 SEQUENCE { |  |  |  |
| supportOfRedCap-r17 | Checked |  | pc_supportOfRedCap_r17 |
| supportOf16DRB-RedCap-r17 | Checked | Acc. to TS 38.306 [23] clause 4.2.21.2 “This capability is only applicable for RedCap UEs.” | pc_16DRB_RedCap _r17 |
| } |  |  |  |
| ra-SDT-r17 | Checked |  | pc_ra_SDT_r17 |
| srb-SDT-r17 | Checked |  | pc_srb_SDT_r17 |
| gNB-SideRTT-BasedPDC-r17 | Not checked |  |  |
| bh-RLF-DetectionRecovery-Indication-r17 | Not checked |  |  |
| nrdc-Parameters-v1700 | Not checked |  |  |
| bap-Parameters-v1700 | Not checked |  |  |
| musim-GapPreference-r17 | Checked |  | pc_musim_GapPreference_r17 |
| musimLeaveConnected-r17 | Checked |  | pc_musimLeaveConnected_r17 |
| mbs-Parameters-r17 | Not checked |  |  |
| nonTerrestrialNetwork-r17 | Checked |  | pc_nonTerrestrialNetwork_r17 |
| ntn-ScenarioSupport-r17 | Not checked |  |  |
| sliceInfoforCellReselection-r17 | Checked |  | pc_sliceInfoforCellReselection_r17 |
| ue-RadioPagingInfo-r17 SEQUENCE { |  |  |  |
| pei-SubgroupingSupportBandList-r17 | Checked | Checked if any frequency band is reported | pc_pei_SubgroupingSupportBandList_r17 |
| } |  |  |  |
| ul-GapFR2-Pattern-r17 | Not checked |  |  |
| ntn-Parameters-r17 | Not checked |  |  |
| nonCriticalExtension |  | UE-NR-Capability-v1740 |  |
| redCapParameters-v1740 SEQUENCE { |  |  |  |
| ncd-SSB-ForRedCapInitialBWP-SDT-r17 | Checked |  | pc_ncd_SSB_ForRedCapInitialBWP_SDT_r17 |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  | UE-NR-Capability-v1750 |  |
| crossCarrierSchedulingConfigurationRelease-r17 | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  | UE-NR-Capability-v1800 |  |
| airToGroundNetwork-r18 | Checked |  | pc_airToGroundNetwork_r18 |
| eRedCapParameters-r18 SEQUENCE { |  | ERedCapParameters-r18 |  |
| supportOfERedCap-r18 | Checked |  | pc_supportOfERedCap_r18 |
| eRedCapNotReducedBB-BW-r18 | Checked |  | pc_eRedCapNotReducedBB_BW_r18 |
| eRedCapIgnoreCapabilityFiltering-r18 | Not checked |  |  |
| } |  |  |  |
| ncr-Parameters-r18 | Not checked |  |  |
| softSatelliteSwitchResyncNTN-r18 | Not checked |  |  |
| hardSatelliteSwitchResyncNTN-r18 | Not checked |  |  |
| mt-SDT-r18 | Checked |  | pc_mt_SDT_r18 |
| mt-SDT-NTN-r18 | Not checked |  |  |
| inDeviceCoexIndAutonomousDenial-r18 | Not checked |  |  |
| inDeviceCoexIndFDM-r18 | Checked |  | pc_inDeviceCoexIndFDM_r18 |
| inDeviceCoexIndTDM-r18 | Checked |  | pc_inDeviceCoexIndTDM_r18 |
| musim-GapPriorityPreference-r18 | Not checked |  |  |
| musim-CapabilityRestriction-r18 | Checked |  | pc_musim_CapabilityRestriction_r18 |
| dummy | Not checked |  |  |
| ra-InsteadCG-SDT-r18 | Checked |  | pc_ra_InsteadCG_SDT_r18 |
| resumeAfterSDT-Release-r18 | Not checked |  |  |
| ul-TrafficInfo-r18 | Checked |  | pc_ul_TrafficInfo_r18 |
| aerialParameters-r18 | Not checked |  |  |
| ntn-VSAT-AntennaType-r18 | Not checked |  |  |
| ntn-VSAT-MobilityType-r18 | Not checked |  |  |
| ntn-Parameters-v1820 | Not checked |  |  |
| nonCriticalExtension SEQUENCE {} |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| Note 1: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and intraAndInterF-MeasAndReport is supported in both modes, then support of intraAndInterF-MeasAndReport will be signalled in measAndMobParameters/measAndMobParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and intraAndInterF-MeasAndReport is only supported in one mode, then support of intraAndInterF-MeasAndReport will be signalled in one of fdd-Add-UE-NR-Capabilities/measAndMobParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/measAndMobParametersXDD-Diff as appropriate. Note 2: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and eventA-MeasAndReport is supported in both modes, then support of eventA-MeasAndReport will be signalled in measAndMobParameters/measAndMobParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and eventA-MeasAndReport is only supported in one mode, then support of eventA-MeasAndReport will be signalled in one of fdd-Add-UE-NR-Capabilities/measAndMobParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/measAndMobParametersXDD-Diff as appropriate. Note 3: If the UE supports single frequency range (FR1 or FR2), or the UE supports both frequency ranges (FR1 and FR2) and csi-RSRP-AndRSRQ-MeasWithSSB is supported in both frequency ranges, then support of csi-RSRP-AndRSRQ-MeasWithSSB will be signaled in measAndMobParameters/measAndMobParametersFRX-Diff. If the UE supports both frequency ranges (FR1 + FR2) and csi-RSRP-AndRSRQ-MeasWithSSB is only supported in one frequency range, then support of csi-RSRP-AndRSRQ-MeasWithSSB will be signaled in one of fr1-Add-UE-NR-Capabilities/measAndMobParametersFRX-Diff or fr2-Add-UE-NR-Capabilities/measAndMobParametersFRX-Diff as appropriate. Note 4: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and skipUplinkTxDynamic is supported in both modes, then support of skipUplinkTxDynamic will be signalled in mac-Parameters/mac-ParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and skipUplinkTxDynamic is only supported in one mode, then skipUplinkTxDynamic will be signalled in one of fdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff as appropriate. Note 5: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and logicalChannelSR-DelayTimer is supported in both modes, then support of logicalChannelSR-DelayTimer will be signalled in mac-Parameters/mac-ParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and logicalChannelSR-DelayTimer is only supported in one mode, then support of logicalChannelSR-DelayTimer will be signalled in one of fdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff as appropriate. Note 6: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and longDRX_Cycle is supported in both modes, then support of longDRX_Cycle will be signalled in mac-Parameters/mac-ParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and longDRX_Cycle is only supported in one mode, then support of longDRX_Cycle will be signalled in one of fdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff as appropriate. Note 7: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and shortDRX_Cycle is supported in both modes, then support of shortDRX_Cycle will be signalled in mac-Parameters/mac-ParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and shortDRX_Cycle is only supported in one mode, then support of shortDRX_Cycle will be signalled in one of fdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff as appropriate. Note 8: If the UE supports FR2 and pc_pdsch_256QAM_FR2 is supported, then support of pdsch_256QAM_FR2 will be signalled in at least one entry of supportedBandListNR as appropriate. Note 9: If pc_pusch_256QAM_FR1 or pc_pusch_256QAM_FR2 is supported, then support of pusch-256QAM will be signalled in at least one entry of supportedBandListNR as appropriate. Note 10: If the UE supports single frequency range (FR1 or FR2), or the UE supports both frequency ranges (FR1 and FR2) and ss-SINR-Meas is supported in both frequency ranges, then support of ss-SINR-Meas will be signaled in measAndMobParameters/measAndMobParametersFRX-Diff. If the UE supports both frequency ranges (FR1 + FR2) and ss-SINR-Meas is only supported in one frequency range, then support of ss-SINR-Meas will be signaled in one of fr1-Add-UE-NR-Capabilities/measAndMobParametersFRX-Diff or fr2-Add-UE-NR-Capabilities/measAndMobParametersFRX-Diff as appropriate. Note 11: If the UE supports single frequency range (FR1 or FR2), or the UE supports both frequency ranges (FR1 and FR2) and csi-RSRP-AndRSRQ-MeasWithoutSSB is supported in both frequency ranges, then support of csi-RSRP-AndRSRQ-MeasWithoutSSB will be signaled in measAndMobParameters/measAndMobParametersFRX-Diff. If the UE supports both frequency ranges (FR1 + FR2) and csi-RSRP-AndRSRQ-MeasWithoutSSB is only supported in one frequency range, then support of csi-RSRP-AndRSRQ-MeasWithoutSSB will be signaled in one of fr1-Add-UE-NR-Capabilities/measAndMobParametersFRX-Diff or fr2-Add-UE-NR-Capabilities/measAndMobParametersFRX-Diff as appropriate. Note 12: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and multipleConfiguredGrants is supported in both modes, then support of multipleConfiguredGrants will be signaled in mac-Parameters/mac-ParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and multipleConfiguredGrants is only supported in one mode, then support of multipleConfiguredGrants will be signaled in one of fdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/mac-ParametersXDD-Diff as appropriate. Note 13: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and sftd-MeasNR-Neigh is supported in both modes, then support of sftd-MeasNR-Neigh will be signaled in measAndMobParameters/measAndMobParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and sftd-MeasNR-Neigh is only supported in one mode, then support of sftd-MeasNR-Neigh will be signaled in one of fdd-Add-UE-NR-Capabilities/measAndMobParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/measAndMobParametersXDD-Diff as appropriate. Note 14: If the UE supports single frequency range (FR1 or FR2), or the UE supports both frequency ranges (FR1 and FR2) and handoverInterF is supported in both frequency ranges, then support of handoverInterF will be signaled in measAndMobParameters/measAndMobParametersFRX-Diff. If the UE supports both frequency ranges (FR1 + FR2) and handoverInterF is only supported in one frequency range, then support of handoverInterF will be signaled in one of fr1-Add-UE-NR-Capabilities/measAndMobParametersFRX-Diff or fr2-Add-UE-NR-Capabilities/measAndMobParametersFRX-Diff as appropriate. Note 15: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and handoverInterF is supported in both modes, then support of handoverInterF will be signaled in measAndMobParameters/measAndMobParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and handoverInterF is only supported in one mode, then support of handoverInterF will be signaled in one of fdd-Add-UE-NR-Capabilities/measAndMobParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/measAndMobParametersXDD-Diff as appropriate. Note 16: If the UE supports single frequency range (FR1 or FR2), or the UE supports both frequency ranges (FR1 and FR2) and voiceOverNR is supported in both frequency ranges, then support of voiceOverNR will be signaled in ims-Parameters/ims-ParametersFRX-Diff. If the UE supports both frequency ranges (FR1 + FR2) and voiceOverNR is only supported in one frequency range, then support of voiceOverNR will be signaled in one of fr1-Add-UE-NR-Capabilities-v1540/ims-ParametersFRX-Diff or fr2-Add-UE-NR-Capabilities-v1540/ims-ParametersFRX-Diff as appropriate. Note 17: If the UE is single mode (FDD or TDD), or the UE is dual mode (FDD and TDD) and dl-SchedulingOffset-PDSCH-TypeA is supported in both modes, then support of dl-SchedulingOffset-PDSCH-TypeA will be signaled in phy-Parameters/phy-ParametersXDD-Diff. If the UE is dual mode (FDD + TDD) and dl-SchedulingOffset-PDSCH-TypeA is only supported in one mode, then support of dl-SchedulingOffset-PDSCH-TypeA will be signaled in one of fdd-Add-UE-NR-Capabilities/ phy-ParametersXDD-Diff or tdd-Add-UE-NR-Capabilities/phy-ParametersXDD-Diff as appropriate. Note 18: If the UE supports single frequency range (FR1 or FR2), or the UE supports both frequency ranges (FR1 and FR2) and dl-SchedulingOffset-PDSCH-TypeA is supported in both frequency ranges, then support of dl-SchedulingOffset-PDSCH-TypeA will be signaled in phy-Parameters/phy-ParametersFRX-Diff. If the UE supports both frequency ranges (FR1 + FR2) and dl-SchedulingOffset-PDSCH-TypeA is only supported in one frequency range, then support of dl-SchedulingOffset-PDSCH-TypeA will be signaled in one of fr1-Add-UE-NR-Capabilities/phy-ParametersFRX-Diff or fr2-Add-UE-NR-Capabilities/phy-ParametersFRX-Diff as appropriate. Note 19: If the UE supports single frequency range (FR1 or FR2), or the UE supports both frequency ranges (FR1 and FR2) and drx-Adaptation-r16 is supported in both frequency ranges, then support of drx-Adaptation-r16 will be signaled in UE-NR-Capability-v1610/mac-Parameters-v1610. If the UE supports both frequency ranges (FR1 and FR2) and the support of drx-Adaptation-r16 in MAC-ParametersFRX-Diff-r16 are different in both frequency ranges, then support of drx-Adaptation-r16 will be signaled under UE-NR-Capability-v1610/fr1-Add-UE-NR-Capabilities-v1610/mac-ParametersFRX-Diff-r16 and UE-NR-Capability-v1610/fr2-Add-UE-NR-Capabilities-v1610/mac-ParametersFRX-Diff-r16, If the UE supports both frequency ranges (FR1 + FR2) and drx-Adaptation-r16 is only supported in one frequency range, then support of drx-Adaptation-r16 will be signaled in one of UE-NR-Capability-v1610/fr1-Add-UE-NR-Capabilities-v1610 or UE-NR-Capability-v1610/fr2-Add-UE-NR-Capabilities-v1610 as appropriate. Note 20: If UE supports FR1 and pc_bwp_DiffNumerology_FR1_FDD or pc_bwp_DiffNumerology_FR1_TDD or supports FR2 and pc_bwp_DiffNumerology_FR2, then support of bwp-DiffNumerology will be signaled in at least one entry of supportedBandListNR as appropriate. Note 21: If UE supports FR1 and pc_bwp_SameNumerology_upto2_FR1_FDD or pc_bwp_SameNumerology_upto2_FR1_TDD or pc_bwp_SameNumerology_upto4_FR1_FDD or pc_bwp_SameNumerology_upto4_FR1_TDD or supports FR2 and pc_bwp_SameNumerology_upto2_FR2 or pc_bwp_SameNumerology_upto4_FR2 then support of bwp_SameNumerology will be signaled in at least one entry of supportedBandListNR as appropriate. Note 22: If the UE supports single frequency range (FR1 or FR2), or the UE supports both frequency ranges (FR1 and FR2) and idleInactiveNR-MeasReport-r16 is supported in both frequency ranges, then support of idleInactiveNR-MeasReport-r16 will be signaled in measAndMobParameters/measAndMobParametersFRX-Diff. If the UE supports both frequency ranges (FR1 + FR2) and idleInactiveNR-MeasReport-r16 is only supported in one frequency range, then support of idleInactiveNR-MeasReport-r16 will be signaled in one of fr1-Add-UE-NR-Capabilities/measAndMobParametersFRX-Diff or fr2-Add-UE-NR-Capabilities/measAndMobParametersFRX-Diff as appropriate. Note 23: If the UE supports single frequency range (FR1 or FR2), or the UE supports both frequency ranges (FR1 and FR2) and directMCG-SCellActivation-r16 is supported in both frequency ranges, then support of directMCG-SCellActivation-r16 will be signaled in UE-NR-Capability-v1610/mac-Parameters-v1610. If the UE supports both frequency ranges (FR1 + FR2) and directMCG-SCellActivation-r16 is only supported in one frequency range, then support of directMCG-SCellActivation-r16 will be signaled in one of UE-NR-Capability-v1610/fr1-Add-UE-NR-Capabilities-v1610 or UE-NR-Capability-v1610/fr2-Add-UE-NR-Capabilities-v1610 as appropriate. Note 24: If the UE supports single frequency range (FR1 or FR2), or the UE supports both frequency ranges (FR1 and FR2) and directMCG-SCellActivationResume-r16 is supported in both frequency ranges, then support of directMCG-SCellActivationResume-r16 will be signaled in UE-NR-Capability-v1610/mac-Parameters-v1610. If the UE supports both frequency ranges (FR1 + FR2) and directMCG-SCellActivationResume-r16 is only supported in one frequency range, then support of directMCG-SCellActivationResume-r16 will be signaled in one of UE-NR-Capability-v1610/fr1-Add-UE-NR-Capabilities-v1610 or UE-NR-Capability-v1610/fr2-Add-UE-NR-Capabilities-v1610 as appropriate. Note 25: If the UE supports single frequency range (FR1 or FR2), or the UE supports both frequency ranges (FR1 and FR2) and directSCG-SCellActivationResume-r16 is supported in both frequency ranges, then support of directSCG-SCellActivationResume-r16 will be signaled in UE-NR-Capability-v1610/mac-Parameters-v1610. If the UE supports both frequency ranges (FR1 + FR2) and directSCG-SCellActivationResume-r16 is only supported in one frequency range, then support of directSCG-SCellActivationResume-r16 will be signaled in one of UE-NR-Capability-v1610/fr1-Add-UE-NR-Capabilities-v1610 or UE-NR-Capability-v1610/fr2-Add-UE-NR-Capabilities-v1610 as appropriate. Note 26: If the UE supports single frequency range (FR1 or FR2), or the UE supports both frequency ranges (FR1 and FR2) and twoPUCCH-AnyOthersInSlot is supported in both frequency ranges, then support of twoPUCCH-AnyOthersInSlot will be signaled in phy-Parameters/phy-ParametersFRX-Diff. If the UE supports both frequency ranges (FR1 + FR2) and twoPUCCH-AnyOthersInSlot is only supported in one frequency range, then support of twoPUCCH-AnyOthersInSlot will be signaled in one of fr1-Add-UE-NR-Capabilities/phy-ParametersFRX-Diff or fr2-Add-UE-NR-Capabilities/phy-ParametersFRX-Diff as appropriate. Note 27: If the UE supports single frequency range (FR1 or FR2), or the UE supports both frequency ranges (FR1 and FR2) and directSCG-SCellActivation-r16 is supported in both frequency ranges, then support of directSCG-SCellActivation-r16 will be signaled in UE-NR-Capability-v1610/mac-Parameters-v1610. If the UE supports both frequency ranges (FR1 + FR2) and directSCG-SCellActivation-r16 is only supported in one frequency range, then support of directSCG-SCellActivation-r16 will be signaled in one of UE-NR-Capability-v1610/fr1-Add-UE-NR-Capabilities-v1610 or UE-NR-Capability-v1610/fr2-Add-UE-NR-Capabilities-v1610 as appropriate. |  |  |  |

Table 8.1.5.1.1.3.3-4: UECapabilityEnquiry-v1560-IEs (Table 8.1.5.1.1.3.3-1)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| UECapabilityEnquiry-v1560-IEs ::= SEQUENCE { |  |  |  |
| capabilityRequestFilterCommon | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rrc-SegAllowed-r16 | enabled |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.1.1.3.3-5: ULDedicatedMessageSegment (step 2a1, Table 8.1.5.1.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ULDedicatedMessageSegment-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ulDedicatedMessageSegment-r16 SEQUENCE { |  |  |  |
| segmentNumber-r16 | 0 for first segment, and incremented by 1 for each subsequent segment |  |  |
| rrc-MessageSegmentContainer-r16 | Not Checked | OCTET STRING including segmented UECapabilityInformation message |  |
| rrc-MessageSegmentType-r16 | notLastSegment for all segments except the last segment. lastSegment for the last segment |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.1.1.3.3-6: FeatureSetDownlink-v1610 (Table 8.1.5.1.1.3.3-3)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| FeatureSetDownlink-v1610 ::= SEQUENCE { |  |  |  |
| cbgPDSCH-ProcessingType1-DifferentTB-PerSlot-r16 | Not checked |  |  |
| cbgPDSCH-ProcessingType2-DifferentTB-PerSlot-r16 | Not checked |  |  |
| intraFreqDAPS-r16 | Checked |  | pc_intraFreqDAPS_r16 |
| intraBandFreqSeparationDL-v1620 | Not checked |  |  |
| intraBandFreqSeparationDL-Only-r16 | Not checked |  |  |
| pdcch-Monitoring-r16 | Not checked |  |  |
| pdcch-MonitoringMixed-r16 | Not checked |  |  |
| crossCarrierSchedulingProcessing-DiffSCS-r16 | Not checked |  |  |
| singleDCI-SDM-scheme-r16 | Not checked |  |  |
| } |  |  |  |

Table 8.1.5.1.1.3.3-7: FeatureSetUplink-v1610 (Table 8.1.5.1.1.3.3-3)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| FeatureSetUplink-v1610 ::= SEQUENCE { |  |  |  |
| pusch-RepetitionTypeB-r16 | Checked |  | pc_pusch_RepetitionTypeB_r16 |
| ul-CancellationSelfCarrier-r16 | Not checked |  |  |
| ul-CancellationCrossCarrier-r16 | Not checked |  |  |
| ul-FullPwrMode2-MaxSRS-ResInSet-r16 | Not checked |  |  |
| cbgPUSCH-ProcessingType1-DifferentTB-PerSlot-r16 | Not checked |  |  |
| cbgPUSCH-ProcessingType2-DifferentTB-PerSlot-r16 | Not checked |  |  |
| supportedSRS-PosResources-r16 | Not checked |  |  |
| intraFreqDAPS-UL-r16 | Not checked |  |  |
| intraBandFreqSeparationUL-v1620 | Not checked |  |  |
| multiPUCCH-r16 | Not checked |  |  |
| twoPUCCH-Type1-r16 | Not checked |  |  |
| twoPUCCH-Type2-r16 | Not checked |  |  |
| twoPUCCH-Type3-r16 | Not checked |  |  |
| twoPUCCH-Type4-r16 | Not checked |  |  |
| mux-SR-HARQ-ACK-r16 | Not checked |  |  |
| dummy1 | Not checked |  |  |
| dummy2 | Not checked |  |  |
| twoPUCCH-Type5-r16 | Not checked |  |  |
| twoPUCCH-Type6-r16 | Not checked |  |  |
| twoPUCCH-Type7-r16 | Not checked |  |  |
| twoPUCCH-Type8-r16 | Not checked |  |  |
| twoPUCCH-Type9-r16 | Not checked |  |  |
| twoPUCCH-Type10-r16 | Not checked |  |  |
| twoPUCCH-Type11-r16 | Not checked |  |  |
| ul-IntraUE-Mux-r16 | Not checked |  |  |
| ul-FullPwrMode-r16 | Checked |  | pc_ul_FullPwrMode_r16 |
| crossCarrierSchedulingProcessing-DiffSCS-r16 | Not checked |  |  |
| ul-FullPwrMode1-r16 | Checked |  | pc_ul_FullPwrMode1_r16 |
| ul-FullPwrMode2-SRSConfig-diffNumSRSPorts-r16 | Not checked |  |  |
| ul-FullPwrMode2-TPMIGroup-r16 | Not checked |  |  |
| } |  |  |  |

Table 8.1.5.1.1.3.3-7a: FeatureSetDownlinkPerCC-v1620 (Table 8.1.5.1.1.3.3-3)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| FeatureSetDownlinkPerCC-v1620 ::= SEQUENCE { |  |  |  |
| multiDCI-MultiTRP-r16 | Checked |  | pc_multiDCI_MultiTRP_r16 |
| supportFDM-SchemeB-r16 | Not checked |  |  |
| } |  |  |  |

Table 8.1.5.1.1.3.3-7b: FeatureSetUplink-v1710 (Table 8.1.5.1.1.3.3-3)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| FeatureSetUplink-v1710 ::= SEQUENCE { |  |  |  |
| mTRP-PUSCH-TypeA-CB-r17 | Checked |  | pc_mtrpPuschTypeACb_r17 |
| mTRP-PUSCH-RepetitionTypeA-r17 | Not checked |  |  |
| mTRP-PUCCH-IntraSlot-r17 | Not checked |  |  |
| srs-AntennaSwitching2SP-1Periodic-r17 | Not checked |  |  |
| srs-ExtensionAperiodicSRS-r17 | Not checked |  |  |
| srs-OneAP-SRS-r17 | Not checked |  |  |
| ue-PowerClassPerBandPerBC-r17 | Not checked |  |  |
| tx-Support-UL-GapFR2-r17 | Not checked |  |  |
| } |  |  |  |

Table 8.1.5.1.1.3.3-7c: FeatureSetUplinkPerCC-v1700 (Table 8.1.5.1.1.3.3-3)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| FeatureSetUplinkPerCC-v1700 ::= SEQUENCE { |  |  |  |
| supportedMinBandwidthUL-r17 | Not checked |  |  |
| mTRP-PUSCH-RepetitionTypeB-r17 | Not checked |  |  |
| mTRP-PUSCH-TypeB-CB-r17 | Checked |  | pc_mtrpPuschTypeBCb_r17 |
| supportedBandwidthUL-v1710 | Not checked |  |  |
| } |  |  |  |

Table 8.1.5.1.1.3.3-7d: FeatureSetDownlinkPerCC-v1700 (Table 8.1.5.1.1.3.3-3)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| FeatureSetDownlinkPerCC-v1700 ::= SEQUENCE { |  |  |  |
| supportedMinBandwidthDL-r17 | Not checked |  |  |
| broadcastSCell-r17 | Checked |  | pc_broadcastSCell_r17 |
| maxNumberMIMO-LayersMulticastPDSCH-r17 | Not checked |  |  |
| dynamicMulticastSCell-r17 | Not checked |  |  |
| supportedBandwidthDL-v1710 | Not checked |  |  |
| supportedCRS-InterfMitigation-r17 | Not checked |  |  |
| } |  |  |  |

Table 8.1.5.1.1.3.3-8: CA-ParametersNR-v1610 (Table 8.1.5.1.1.3.3-3)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| CA-ParametersNR-v1610 ::= SEQUENCE { |  |  |  |
| parallelTxMsgA-SRS-PUCCH-PUSCH-r16 | Not checked |  |  |
| msgA-SUL-r16 | Not checked |  |  |
| jointSearchSpaceSwitchAcrossCells-r16 | Not checked |  |  |
| half-DuplexTDD-CA-SameSCS-r16 | Not checked |  |  |
| scellDormancyWithinActiveTime-r16 | Not checked |  |  |
| scellDormancyOutsideActiveTime-r16 | Checked |  | pc_scellDormancyOutsideActiveTime_r16 |
| crossCarrierA-CSI-trigDiffSCS-r16 | Not checked |  |  |
| defaultQCL-CrossCarrierA-CSI-Trig-r16 | Not checked |  |  |
| interCA-NonAlignedFrame-r16 | Not checked |  |  |
| simul-SRS-Trans-BC-r16 | Not checked |  |  |
| interFreqDAPS-r16 | Checked |  | pc_interFreqDAPS_r16 |
| codebookParametersPerBC-r16 | Not checked |  |  |
| blindDetectFactor-r16 | Not checked |  |  |
| pdcch-MonitoringCA-r16 | Not checked |  |  |
| pdcch-BlindDetectionCA-Mixed-r16 | Not checked |  |  |
| pdcch-BlindDetectionMCG-UE-r16 | Not checked |  |  |
| pdcch-BlindDetectionSCG-UE-r16 | Not checked |  |  |
| pdcch-BlindDetectionMCG-UE-Mixed-r16 | Not checked |  |  |
| pdcch-BlindDetectionSCG-UE-Mixed-r16 | Not checked |  |  |
| crossCarrierSchedulingDL-DiffSCS-r16 | Not checked |  |  |
| crossCarrierSchedulingDefaultQCL-r16 | Not checked |  |  |
| crossCarrierSchedulingUL-DiffSCS-r16 | Not checked |  |  |
| simul-SRS-MIMO-Trans-BC-r16 | Not checked |  |  |
| codebookParametersAdditionPerBC-r16 | Not checked |  |  |
| codebookComboParametersAdditionPerBC-r16 | Not checked |  |  |
| } |  |  |  |

Table 8.1.5.1.1.3.3-9: PowSav-Parameters-r16 (Table 8.1.5.1.1.3.3-3)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| PowSav-Parameters-r16 ::= SEQUENCE { |  |  |  |
| powSav-ParametersCommon-r16 SEQUENCE { |  |  |  |
| drx-Preference-r16 | Not checked |  |  |
| maxCC-Preference-r16 | Not checked |  |  |
| releasePreference-r16 | Checked |  | pc_releasePreference_r16 |
| minSchedulingOffsetPreference-r16 | Not checked |  |  |
| } |  |  |  |
| powSav-ParametersFRX-Diff-r16 | Not checked |  |  |
| } |  |  |  |

Table 8.1.5.1.1.3.3-10: UE-BasedPerfMeas-Parameters-r16 (Table 8.1.5.1.1.3.3-3)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| UE-BasedPerfMeas-Parameters-r16 ::= SEQUENCE { |  |  |  |
| barometerMeasReport-r16 | Checked |  | pc_barometer_r16 |
| immMeasBT-r16 | Checked |  | pc_immMeasBT_r16 |
| immMeasWLAN-r16 | Checked |  | pc_immMeasWLAN_r16 |
| loggedMeasBT-r16 | Checked |  | pc_loggedMeasBT_r16 |
| loggedMeasurements-r16 | Checked |  | pc_loggedMeasurements_r16 |
| loggedMeasWLAN-r16 | Checked |  | pc_loggedMeasWLAN_r16 |
| orientationMeasReport-r16 | Checked |  | pc_orientation_r16 |
| speedMeasReport-r16 | Checked |  | pc_speed_r16 |
| gnss-Location-r16 | Checked |  | pc_GNSS_location_r16 |
| ulPDCP-Delay-r16 | Checked |  | pc_PDCP_Delay_r16 |
| sigBasedLogMDT-OverrideProtect-r17 | Not checked |  |  |
| multipleCEF-Report-r17 | Checked |  | pc_multiple_CEF_Report_r17 |
| excessPacketDelay-r17 | Not checked |  |  |
| earlyMeasLog-r17 | Checked |  | pc_earlyMeasLog_r17 |
| } |  |  |  |

Table 8.1.5.1.1.3.3-11: MAC-ParametersFRX-Diff-r16 (Table 8.1.5.1.1.3.3-3)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| MAC-ParametersFRX-Diff-r16 ::= SEQUENCE { |  |  |  |
| directMCG-SCellActivation-r16 | Checked |  | pc_directMCG_SCellActivation_r16 |
| directMCG-SCellActivationResume-r16 | Checked |  | pc_directMCG_SCellActivationResume_r16 |
| directSCG-SCellActivation-r16 | Checked |  | pc_directSCG_SCellActivation_r16 |
| directSCG-SCellActivationResume-r16 | Checked |  | pc_directSCG_SCellActivationResume_r16 |
| drx-Adaptation-r16 | Checked | MinTimeGap-r16 Acc. to TS 38.306 [23] clause 4.2.6: when this field is reported, either of sharedSpectrumChAccess-r16 or non-SharedSpectrumChAccess-r16 shall be reported, at least. | pc_DRX_Adaptation |
| } |  |  |  |

Table 8.1.5.1.1.3.3-12: MIMO-ParametersPerBand (Table 8.1.5.1.1.3.3-3)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| MIMO-ParametersPerBand ::= SEQUENCE { | Not checked |  |  |
| tci-StatePDSCH | Not checked |  |  |
| additionalActiveTCI-StatePDCCH | Not checked |  |  |
| pusch-TransCoherence | Not checked |  |  |
| beamCorrespondenceWithoutUL-BeamSweeping | Not checked |  |  |
| periodicBeamReport | Not checked |  |  |
| aperiodicBeamReport | Not checked |  |  |
| sp-BeamReportPUCCH | Not checked |  |  |
| sp-BeamReportPUSCH | Not checked |  |  |
| dummy1 | Not checked |  |  |
| maxNumberRxBeam | Not checked |  |  |
| maxNumberRxTxBeamSwitchDL | Not checked |  |  |
| maxNumberNonGroupBeamReporting | Not checked |  |  |
| groupBeamReporting | Not checked |  |  |
| uplinkBeamManagement | Not checked |  |  |
| maxNumberCSI-RS-BFD | Checked | Acc. to TS 38.306 [23] clause 4.2.7.2: In this release, the maximum value that can be signalled is 16 [,,] It is mandatory with capability signalling for FR2 and optional for FR1. | pc_maxNumberCSI_RS_BFD |
| maxNumberSSB-BFD | Not checked |  |  |
| maxNumberCSI-RS-SSB-CBD | Checked | Acc. to TS 38.306 [23] clause 4.2.7.2: In this release, the maximum value that can be signalled is 128. [,,] It is mandatory with capability signalling for FR2 and optional for FR1. The UE is mandated to report at least 32 for FR2. | pc_maxNumberCSI_RS_SSB_CBD |
| dummy2 |  |  |  |
| twoPortsPTRS-UL |  |  |  |
| dummy5 |  |  |  |
| dummy3 |  |  |  |
| beamReportTiming |  |  |  |
| ptrs-DensityRecommendationSetDL |  |  |  |
| ptrs-DensityRecommendationSetUL |  |  |  |
| dummy4 |  |  |  |
| aperiodicTRS |  |  |  |
| srs-increasedRepetition-r17 | Not checked |  |  |
| srs-partialFrequencySounding-r17 | Checked |  | pc_srs_partialFrequencySounding_r17 |
| srs-startRB-locationHoppingPartial-r17 | Checked |  | pc_srs_startRB_locationHoppingPartial_r17 |
| srs-combEight-r17 | Not checked |  |  |
| codebookParametersfetype2-r17 | Not checked |  |  |
| mTRP-PUSCH-twoCSI-RS-r17 | Not checked |  |  |
| mTRP-PUCCH-InterSlot-r17 | Not checked |  |  |
| mTRP-PUCCH-CyclicMapping-r17 | Not checked |  |  |
| mTRP-PUCCH-SecondTPC-r17 | Not checked |  |  |
| mTRP-BFR-twoBFD-RS-Set-r17 | Not checked |  |  |
| mTRP-BFR-PUCCH-SR-perCG-r17 | Not checked |  |  |
| mTRP-BFR-association-PUCCH-SR-r17 | Not checked |  |  |
| sfn-SimulTwoTCI-AcrossMultiCC-r17 | Not checked |  |  |
| sfn-DefaultDL-BeamSetup-r17 | Not checked |  |  |
| sfn-DefaultUL-BeamSetup-r17 | Not checked |  |  |
| srs-TriggeringOffset-r17 | Not checked |  |  |
| srs-TriggeringDCI-r17 | Not checked |  |  |
| codebookComboParameterMixedType-r17 | Not checked |  |  |
| unifiedJointTCI-r17 | Checked |  | pc_unifiedJointTCI_r17 |
| unifiedJointTCI-multiMAC-CE-r17 | Not checked |  |  |
| unifiedJointTCI-perBWP-CA-r17 | Not checked |  |  |
| unifiedJointTCI-ListSharingCA-r17 | Not checked |  |  |
| unifiedJointTCI-commonMultiCC-r17 | Not checked |  |  |
| unifiedJointTCI-BeamAlignDLRS-r17 | Not checked |  |  |
| unifiedJointTCI-PC-association-r17 | Not checked |  |  |
| unifiedJointTCI-Legacy-r17 | Not checked |  |  |
| unifiedJointTCI-Legacy-SRS-r17 | Not checked |  |  |
| unifiedJointTCI-Legacy-CORESET0-r17 | Not checked |  |  |
| unifiedJointTCI-SCellBFR-r17 | Not checked |  |  |
| unifiedJointTCI-InterCell-r17 | Not checked |  |  |
| unifiedSeparateTCI-r17 | Not checked |  |  |
| unifiedSeparateTCI-multiMAC-CE-r17 | Checked |  | pc_unifiedSeparateTCI_multiMAC_CE_r17 |
| unifiedSeparateTCI-perBWP-CA-r17 | Not checked |  |  |
| unifiedSeparateTCI-ListSharingCA-r17 | Not checked |  |  |
| unifiedSeparateTCI-commonMultiCC-r17 | Not checked |  |  |
| unifiedSeparateTCI-InterCell-r17 | Not checked |  |  |
| unifiedJointTCI-mTRP-InterCell-BM-r17 | Not checked |  |  |
| mpe-Mitigation-r17 | Not checked |  |  |
| srs-PortReport-r17 | Not checked |  |  |
| mTRP-PDCCH-individual-r17 | Not checked |  |  |
| mTRP-PDCCH-anySpan-3Symbols-r17 | Not checked |  |  |
| mTRP-PDCCH-TwoQCL-TypeD-r17 | Not checked |  |  |
| mTRP-PUSCH-CSI-RS-r17 | Not checked |  |  |
| mTRP-PUSCH-cyclicMapping-r17 | Checked |  | pc_mtrpPuschCyclicMapping_r17 |
| mTRP-PUSCH-secondTPC-r17 | Not checked |  |  |
| mTRP-PUSCH-twoPHR-Reporting-r17 | Not checked |  |  |
| mTRP-PUSCH-A-CSI-r17 | Not checked |  |  |
| mTRP-PUSCH-SP-CSI-r17 | Not checked |  |  |
| mTRP-PUSCH-CG-r17 | Not checked |  |  |
| mTRP-PUCCH-MAC-CE-r17 | Not checked |  |  |
| mTRP-PUCCH-maxNum-PC-FR1-r17 | Not checked |  |  |
| mTRP-inter-Cell-r17 | Not checked |  |  |
| mTRP-GroupBasedL1-RSRP-r17 | Not checked |  |  |
| mTRP-BFD-RS-MAC-CE-r17 | Not checked |  |  |
| mTRP-CSI-EnhancementPerBand-r17 | Not checked |  |  |
| codebookComboParameterMultiTRP-r17 | Not checked |  |  |
| mTRP-CSI-additionalCSI-r17 | Not checked |  |  |
| mTRP-CSI-N-Max2-r17 | Not checked |  |  |
| mTRP-CSI-CMR-r17 | Not checked |  |  |
| srs-partialFreqSounding-r17 | Not checked |  |  |
| beamSwitchTiming-v1710 | Not checked |  |  |
| beamSwitchTiming-r17 | Not checked |  |  |
| beamReportTiming-v1710 | Not checked |  |  |
| maxNumberRxTxBeamSwitchDL-v1710 | Not checked |  |  |
| } |  |  |  |

Table 8.1.5.1.1.3.3-13: SharedSpectrumChAccessParamsPerBand-r16 (Table 8.1.5.1.1.3.3-3)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| SharedSpectrumChAccessParamsPerBand-r16 ::= SEQUENCE { |  |  |  |
| ul-DynamicChAccess-r16 | Checked |  | pc_ul_DynamicChAccess_r16 |
| ul-Semi-StaticChAccess-r16 | Checked |  | pc_ul_Semi_StaticChAccess_r16 |
| ssb-RRM-DynamicChAccess-r16 | Not checked |  |  |
| ssb-RRM-Semi-StaticChAccess-r16 | Not checked |  |  |
| mib-Acquisition-r16 | Not checked |  |  |
| ssb-RLM-DynamicChAccess-r16 | Not checked |  |  |
| ssb-RLM-Semi-StaticChAccess-r16 | Not checked |  |  |
| sib1-Acquisition-r16 | Not checked |  |  |
| extRA-ResponseWindow-r16 | Not checked |  |  |
| ssb-BFD-CBD-dynamicChannelAccess-r16 | Not checked |  |  |
| ssb-BFD-CBD-semi-staticChannelAccess-r16 | Not checked |  |  |
| csi-RS-BFD-CBD-r16 | Not checked |  |  |
| ul-ChannelBW-SCell-10mhz-r16 | Not checked |  |  |
| rssi-ChannelOccupancyReporting-r16 | Checked |  | pc_rssi_ChannelOccupancyReporting_r16 |
| srs-StartAnyOFDM-Symbol-r16 | Not checked |  |  |
| searchSpaceFreqMonitorLocation-r16 | Not checked |  |  |
| coreset-RB-Offset-r16 | Not checked |  |  |
| cgi-Acquisition-r16 | Not checked |  |  |
| configuredUL-Tx-r16 | Not checked |  |  |
| prach-Wideband-r16 | Not checked |  |  |
| dci-AvailableRB-Set-r16 | Not checked |  |  |
| dci-ChOccupancyDuration-r16 | Not checked |  |  |
| typeB-PDSCH-length-r16 | Not checked |  |  |
| searchSpaceSwitchWithDCI-r16 | Not checked |  |  |
| searchSpaceSwitchWithoutDCI-r16 | Not checked |  |  |
| searchSpaceSwitchCapability2-r16 | Not checked |  |  |
| non-numericalPDSCH-HARQ-timing-r16 | Not checked |  |  |
| enhancedDynamicHARQ-codebook-r16 | Not checked |  |  |
| oneShotHARQ-feedback-r16 | Not checked |  |  |
| multiPUSCH-UL-grant-r16 | Not checked |  |  |
| csi-RS-RLM-r16 | Not checked |  |  |
| dummy | Not checked |  |  |
| periodicAndSemi-PersistentCSI-RS-r16 | Not checked |  |  |
| pusch-PRB-interlace-r16 | Not checked |  |  |
| pucch-F0-F1-PRB-Interlace-r16 | Not checked |  |  |
| occ-PRB-PF2-PF3-r16 | Not checked |  |  |
| extCP-rangeCG-PUSCH-r16 | Not checked |  |  |
| configuredGrantWithReTx-r16 | Not checked |  |  |
| ed-Threshold-r16 | Not checked |  |  |
| ul-DL-COT-Sharing-r16 | Not checked |  |  |
| mux-CG-UCI-HARQ-ACK-r16 | Not checked |  |  |
| cg-resourceConfig-r16 | Not checked |  |  |
| } |  |  |  |

Table 8.1.5.1.1.3.3-14: CA-ParametersNR-v1800 (Table 8.1.5.1.1.3.3-3)

| Information Element |  | Value/remark |  | Comment |  | Condition |
| --- | --- | --- | --- | --- | --- | --- |
| CA-ParametersNR-v1800 ::= SEQUENCE { |  |  |  |  |  |  |  |
| codebookParametersetype2DopplerCSI-PerBC-r18 |  | Not checked |  |  |  |  |  |
| codebookParametersfetype2DopplerCSI-PerBC-r18 |  | Not checked |  |  |  |  |  |
| codebookParametersetype2CJT-PerBC-r18 |  | Not checked |  |  |  |  |  |
| codebookParametersfetype2CJT-PerBC-r18 |  | Not checked |  |  |  |  |  |
| codebookComboParametersCJT-PerBC-r18 |  | Not checked |  |  |  |  |  |
| codebookParametersHARQ-ACK-PUSCH-PerBC-r18 |  | Not checked |  |  |  |  |  |
| maxNumberTAG-AcrossCC-r18 |  | Not checked |  |  |  |  |  |
| tdcp-ReportPerBC-r18 |  | Not checked |  |  |  |  |  |
| tdcp-ResourcePerBC-r18 |  | Not checked |  |  |  |  |  |
| timelineRelax-CJT-CSI-CA-r18 |  | Not checked |  |  |  |  |  |
| spatialAdaptation-CSI-FeedbackPerBC-r18 |  | Not checked |  |  |  |  |  |
| nonCodebook-CSI-RS-SRS-PerBC-r18 |  | Not checked |  |  |  |  |  |
| spatialAdaptation-CSI-FeedbackPUSCH-PerBC-r18 |  | Not checked |  |  |  |  |  |
| spatialAdaptation-CSI-FeedbackAperiodicPerBC-r18 |  | Not checked |  |  |  |  |  |
| spatialAdaptation-CSI-FeedbackPUCCH-PerBC-r18 |  | Not checked |  |  |  |  |  |
| powerAdaptation-CSI-FeedbackPerBC-r18 |  | Not checked |  |  |  |  |  |
| powerAdaptation-CSI-FeedbackPUSCH-PerBC-r18 |  | Not checked |  |  |  |  |  |
| powerAdaptation-CSI-FeedbackAperiodicPerBC-r18 |  | Not checked |  |  |  |  |  |
| powerAdaptation-CSI-FeedbackPUCCH-PerBC-r18 |  | Not checked |  |  |  |  |  |
| mixCodeBookSpatialAdaptationPerBC-r18 |  | Not checked |  |  |  |  |  |
| simultaneousCSI-SubReportsAllCC-r18 |  | Not checked |  |  |  |  |  |
| multiCell-PDSCH-DCI-1-3-SameSCS-r18 SEQUENCE { |  | Checked |  |  |  | pc_multiCell_PDSCH_DCI_1_3_SameSCS_r18 |  |
| coScheduledCellSCS-r18 |  | Not checked |  |  |  |  |  |
| maxNumberCoScheduledCell-r18 |  | Not checked |  |  |  |  |  |
| maxNumberSetsOfCellAcrossPUCCH-Group-r18 |  | Not checked |  |  |  |  |  |
| maxNumberSetsOfCellScheduling-r18 |  | Not checked |  |  |  |  |  |
| harqFeedbackType-r18 |  | Checked |  |  |  | pc_harqFeedbackType1_DCI_1_3 or pc_harqFeedbackType2_DCI_1_3 |  |
| coScheduledCellIndicationScheme-r18 |  | Checked |  |  |  | pc_coScheduledCellIndicationScheme_r18_fdra_DCI_1_3 and/or pc_coScheduledCellIndicationScheme_r18_cellInd_DCI_1_3 |  |
| supportOfSearchSpace-r18 |  | Checked |  |  |  | pc_supportOfSearchSpace_r18_DCI_1_3 |  |
| licensed-fdd-tdd-fr1-r18 |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| multiCell-PDSCH-DCI-1-3-DiffSCS-r18 |  | Not checked |  |  |  |  |  |
| multiCell-PUSCH-DCI-0-3-SameSCS-r18 SEQUENCE { |  | Checked |  |  |  | pc_multiCell_PUSCH_DCI_0_3_SameSCS_r18 |  |
| coScheduledCellSCS-r18 |  | Not checked |  |  |  |  |  |
| maxNumberSetsOfCellAcrossPUCCH-Group-r18 |  | Not checked |  |  |  |  |  |
| maxNumberSetsOfCellScheduling-r18 |  | Not checked |  |  |  |  |  |
| coScheduledCellIndicationScheme-r18 |  | Checked |  |  |  | pc_coScheduledCellIndicationScheme_r18_fdra_DCI_0_3 and/or pc_coScheduledCellIndicationScheme_r18_cellInd_DCI_0_3 |  |
| supportOfSearchSpace-r18 |  | Checked |  |  |  | pc_supportOfSearchSpace_r18_DCI_0_3 |  |
| licensed-fdd-tdd-fr1-r18 |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| multiCell-PUSCH-DCI-0-3-DiffSCS-r18 |  | Not checked |  |  |  |  |  |
| advUnicastDCI-DL-r18 |  | Not checked |  |  |  |  |  |
| advUnicastDCI-UL-r18 |  | Not checked |  |  |  |  |  |
| type3HARQ-CB-DCI-1-3-r18 |  | Not checked |  |  |  |  |  |
| type3EnhHARQ-CB-DCI-1-3-r18 |  | Not checked |  |  |  |  |  |
| scellDormancyWithinActiveTime-DCI-0-3-And-1-3-r18 |  | Not checked |  |  |  |  |  |
| pdcch-MonitoringCA-Ext-r18 |  | Not checked |  |  |  |  |  |
| pdcch-BlindDetectionCA-MixedExt-r18 |  | Not checked |  |  |  |  |  |
| pdcch-BlindDetectionMCG-SCG-List-r18 |  | Not checked |  |  |  |  |  |
| intraBandNR-CA-non-collocated-r18 |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 8.1.5.1.1.3.3-15: NRDC-Parameters (Table 8.1.5.1.1.3.3-3)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| NRDC-Parameters ::= SEQUENCE { |  |  |  |
| measAndMobParametersNRDC SEQUENCE { |  |  |  |
| measAndMobParametersMRDC_Common SEQUENCE { |  |  |  |
| independentGapConfig | Checked |  | pc_independentGapConfig_NRDC |
| } |  |  |  |
| measAndMobParametersMRDC-XDD-Diff |  |  |  |
| measAndMobParametersMRDC-FRX-Diff |  |  |  |
| } |  |  |  |
| generalParametersNRDC | Not checked |  |  |
| fdd_Add_UE_NRDC_Capabilities | Not checked |  |  |
| tdd_Add_UE_NRDC_Capabilities | Not checked |  |  |
| fr1_Add_UE_NRDC_Capabilities | Not checked |  |  |
| fr2_Add_UE_NRDC_Capabilities | Not checked |  |  |
| dummy2 | Not checked |  |  |
| dummy | Not checked |  |  |
| } |  |  |  |

#### 8.1.5.2	SI change / On-demand SIB

##### 8.1.5.2.1	Void

##### 8.1.5.2.2	SI change / Notification of BCCH modification / Short message for SI update in NR RRC_CONNECTED state

8.1.5.2.2.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE receives a short message transmitted on PDCCH using P-RNTI indicating a systemInfoModification }

then { UE re-acquires the SIB1 }

}

8.1.5.2.2.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.331, clause 5.2.2.2.2, 5.2.2.3.1, 5.2.2.4.2 and 6.5. Unless otherwise stated these are Rel-15 requirements.

[TS 38.331, clause 5.2.2.2.2]

A modification period is used, i.e. updated SI (other than for ETWS and CMAS) is broadcasted in the modification period following the one where SI change indication is transmitted. The modification period boundaries are defined by SFN values for which SFN mod m= 0, where m is the number of radio frames comprising the modification period. The modification period is configured by system information. The UE receives indications about SI modifications and/or PWS notifications using Short Message transmitted with P-RNTI over DCI (see clause 6.5). Repetitions of SI change indication may occur within preceding modification period.

UEs in RRC_IDLE or in RRC_INACTIVE shall monitor for SI change indication in its own paging occasion every DRX cycle. UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.

…

If the UE receives a Short Message, the UE shall:

…

1> if the systemInfoModification bit of Short Message is set:

2>  apply the SI acquisition procedure as defined in sub-clause 5.2.2.3 from the start of the next modification.

[TS 38.331, clause 5.2.2.3.1]

The UE shall:

1>	apply the specified BCCH configuration defined in 9.1.1.1;

…

1>	if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1 and pagingSearchSpace and has received an indication about change of system information; or

…

NOTE:	The UE in RRC_CONNECTED is only required to acquire broadcasted SIB1 if the UE can acquire it without disrupting unicast data reception, i.e. the broadcast and unicast beams are quasi co-located.

[TS 38.331, clause 5.2.2.4.2]

Upon receiving the SIB1 the UE shall:

…

3>	if the UE has a stored valid version of a SIB, in accordance with sub-clause 5.2.2.2.1, that the UE requires to operate within the cell in accordance with sub-clause 5.2.2.1:

4>	use the stored version of the required SIB;

3>	if the UE has not stored a valid version of a SIB, in accordance with sub-clause 5.2.2.2.1, of one or several required SIB(s), in accordance with sub-clause 5.2.2.1:

4>	for the SI message(s) that, according to the si-SchedulingInfo, contain at least one required SIB and for which si-BroadcastStatus is set to broadcasting:

5>	acquire the SI message(s) as defined in sub-clause 5.2.2.3.2;

[TS 38.331, clause 6.5]

Short messages can be transmitted on PDCCH using P-RNTI with or without associated Paging message using Short Message field in DCI format 1_0 (see TS 38.212 [17, 7.3.1.2.1]).

Table 6.5-1 defines Short Messages. Bit 1 is the most significant bit.

Table 6.5-1: Short messages

| Bit | Short message |
| --- | --- |
| 1 | systemInfoModification If set to 1: indication of a BCCH modification other than SIB6, SIB7 and SIB8. |
| 2 | etwsAndCmasIndication If set to 1: indication of an ETWS primary notification and/or an ETWS secondary notification and/or a CMAS notification. |
| 3 – [8] | Not used in this release of the specification, and shall be ignored by UE if received. |

8.1.5.2.2.3	Test Description

8.1.5.2.2.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used in NR cell.

UE:

-	None.

Preamble:

-	The UE is in 5GS state 3N-A and Test Loop Function (ON) with UE test loop mode B established according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3 with the exception that Short_DCI condition is applied in NR Serving cell configuration.

8.1.5.2.2.3.2	Test procedure sequence

Table 8.1.5.2.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS sends one IP Packet to the UE. | - | - | - | - |
| 1A | The SS waits 0.5 second and then ignores scheduling requests and does not allocate any uplink grant. | - | - | - | - |
| 2 | The SS changes the prach-ConfigurationIndex and ra-ResponseWindow in the system information. | - | - | - | - |
| 3 | The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. | - | PDCCH (DCI 1_0): Short Message | - | - |
| 4 | Void | - | - | - | - |
| 5 | Check: Does the UE initiate a random access using prach-ConfigurationIndex given in step 3 to send pending Uplink data? | - | - | 1 | P |
| 6 | The SS transmits Random Access Response with RAPID corresponding to preamble in step 5. (Note 1) | - | - | - | - |
| 7 | The UE loop backs the IP packets received in step 1 | - | - | - | - |
| 8 | The SS starts the UL default grant transmissions. |  |  |  |  |
| Note 1: The resource allocation for Msg3 shall be enough to accommodate all of the DRB data that UE will loop back at step 7. To be more specific, resource allocation shall be no less than: TB size = IP packet (64 bytes) + SDAP header (1 bytes) + PDCP header (3 bytes) + RLC header (3 bytes) + MAC header (2 bytes) + padding or BSR (2 bytes) + C-RNTI MAC CE (3 Bytes) + PHR (3 bytes) = 81 bytes for (pc_supportOfRedCap_r17 =false AND pc_supportOfERedCap_r18= false), or TB size = IP packet (64 bytes) + SDAP header (1 bytes) + PDCP header (2 bytes) + RLC header (2 bytes) + MAC header (2 bytes) + padding and/or BSR (4 bytes) + C-RNTI MAC CE (3 Bytes) + PHR (3 bytes) = 81 bytes for (pc_supportOfRedCap_r17 =true.OR pc_supportOfERedCap_r18=true) |  |  |  |  |  |

8.1.5.2.2.3.3	Specific message contents

Table 8.1.5.2.2.3.3-1: CLOSE UE TEST LOOP (Preamble, Table 8.1.5.2.2.3.2-1)

| Derivation Path: TS 36.508-1 [7] table 4.7A-3 condition UE test loop mode B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE test loop mode B LB setup |  |  |  |
| IP PDU delay | '0000 1111'B | 15 seconds |  |

Table 8.1.5.2.2.3.3-2: SIB1 (step 2, Table 8.1.5.2.2.3.2-1)

| Derivation Path: TS 38.508-1 [4] table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| servingCellConfigCommon SEQUENCE { |  |  |  |
| uplinkConfigCommon SEQUENCE { |  |  |  |
| initialUplinkBWP SEQUENCE { |  |  |  |
| rach-ConfigCommon SEQUENCE { |  |  |  |
| rach-ConfigGeneric SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 119 | . | FR1 FDD |
|  | 120 |  | FR1 FDD AND HD_FDD |
|  | 91 |  | FR1 TDD |
|  | 6 |  | FR2 |
| ra-ResponseWindow | sl20 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| HD_FDD | pc_halfDuplexFDD_TypeA_RedCap_r17 (i.e. HD_FDD UE are performing test on FDD band) |

##### 8.1.5.2.3	eDRX / IDLE / Paging for notification of BCCH modification

8.1.5.2.3.1	Test Purpose (TP)

(1)

with { UE in NR RRC_IDLE state using eDRX cycle longer than the BCCH modification period }

ensure that  {

when {UE receives a Short Message and the systemInfoModification-eDRX bit is set to indicate BCCH modification }

then {UE applies the SI acquisition procedure to update the new system information }

}

(2)

with { UE in NR RRC_IDLE state using eDRX cycle shorter than the BCCH modification period }

ensure that  {

when {UE receives a Short Message and the systemInfoModification bit is set to indicate BCCH modification }

then {UE applies the SI acquisition procedure to update the new system information }

}

8.1.5.2.3.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.331, clause 5.2.2.2.2, ,5.2.2.3.1, 5.2.2.3.2, 5.2.2.4.2 and 6.5, 38.304, clause 7.4. Unless otherwise stated these are Rel-17 requirements.

[TS 38.304, clause 7.4]

The UE may be configured by upper layers and/or RRC with an extended DRX (eDRX) cycle TeDRX, CN and/or TeDRX, RAN. The UE operates in eDRX for CN paging in RRC_IDLE or RRC_INACTIVE states if the UE is configured for eDRX by upper layers and eDRX-AllowedIdle is signalled in SIB1. The UE operates in eDRX for RAN paging in RRC_INACTIVE state if the UE is configured for eDRX by RAN and eDRX-AllowedInactive is signalled in SIB1. If the UE operates in eDRX with an eDRX cycle no longer than 1024 radio frames, it monitors POs as defined in 7.1 with configured eDRX cycle. Otherwise, a UE operating in eDRX monitors POs as defined in 7.1 during a periodic Paging Time Window (PTW) configured for the UE. The PTW is UE-specific and is determined by a Paging Hyperframe (PH), a starting position within the PH (PTW_start) and an ending position (PTW_end). PH, PTW_start and PTW_end are given by the following formula:

The PH for CN is the H-SFN satisfying the following equations:

H-SFN mod TeDRX_CN= (UE_ID_H mod TeDRX_CN), where

-	UE_ID_H: 13 most significant bits of the Hashed ID.

-	TeDRX_CN: UE-specific eDRX cycle in Hyper-frames, (TeDRX_CN = 2, …, 1024 Hyper-frames) configured by upper layers.

PTW_start denotes the first radio frame of the PH that is part of the PTW and has SFN satisfying the following equation:

SFN = 128 * ieDRX_CN, where

-	ieDRX_CN = floor(UE_ID_H /TeDRX_CN) mod 8

PTW_end is the last radio frame of the PTW and has SFN satisfying the following equation:

SFN = (PTW_start + L*100 - 1) mod 1024, where

-	L = Paging Time Window (PTW) length (in seconds) configured by upper layers

Hashed ID is defined as follows:

Hashed_ID is Frame Check Sequence (FCS) for the bits b31, b30…, b0 of 5G-S-TMSI.

5G-S-TMSI = <b47, b46, …, b0> as defined in TS 23.003 [23].

The 32-bit FCS shall be the ones complement of the sum (modulo 2) of Y1 and Y2, where

-	Y1 is the remainder of xk (x31 + x30 + x29 + x28 + x27 + x26 + x25 + x24 + x23 + x22 + x21 + x20 + x19 + x18 + x17 + x16 + x15 + x14 + x13 + x12 + x11 + x10 + x9 + x8 + x7 + x6 + x5 + x4 + x3 + x2 + x1 + 1) divided (modulo 2) by the generator polynomial x32 + x26 + x23 + x22 + x16 + x12 + x11 + x10 + x8 + x7 + x5 + x4 + x2 + x + 1, where k is 32; and

-	Y2 is the remainder of Y3 divided (modulo 2) by the generator polynomial x32 + x26 + x23 + x22 + x16 + x12 + x11 + x10 + x8 + x7 + x5 + x4 + x2 + x + 1, where Y3 is the product of x32 by "b31, b30…, b0 of S-TMSI or 5G-S-TMSI", i.e., Y3 is the generator polynomial x32 (b31*x31 + b30*x30 + … + b0*1).

NOTE:	The Y1 is 0xC704DD7B for any 5G-S-TMSI value. An example of hashed ID calculation is in Annex A.

[TS 38.331, clause 5.2.2.2.2]

A modification period is used, i.e. updated SI message (other than SI message for ETWS, CMAS, positioning assistance data, and some NTN-specific information as specified in the field descriptions ) is broadcasted in the modification period following the one where SI change indication is transmitted. The modification period boundaries are defined by SFN values for which SFN mod m = 0, where m is the number of radio frames comprising the modification period. The modification period is configured by system information. If H-SFN is provided in SIB1, and UE is configured with eDRX, modification period boundaries are defined by SFN values for which (H-SFN * 1024 + SFN) mod m = 0.

For UEs in RRC_IDLE or RRC_INACTIVE configured to use an IDLE eDRX cycle longer than the modification period, an eDRX acquisition period is defined. The boundaries of the eDRX acquisition period are determined by H-SFN values for which H-SFN mod 1024 = 0.

The UE receives indications about SI modifications and/or PWS notifications using Short Message transmitted with P-RNTI over DCI (see clause 6.5). Repetitions of SI change indication may occur within preceding modification period or within preceding eDRX acquisition period. SI change indication is not applicable for SI messages containing posSIBs.

…

For Short Message reception in a paging occasion, the UE monitors the PDCCH monitoring occasion(s) for paging as specified in TS 38.304 [20] and TS 38.213 [13].

A L2 U2N Remote UE is not required to monitor paging occasion for SI modifications and/or PWS notifications. It obtains the updated system information and SIB6/7/8 from the connected L2 U2N Relay UE as defined in clause 5.8.9.9.3.

If the UE receives a Short Message, the UE shall:

…

1>	if the UE does not operate an IDLE eDRX cycle longer than the modification period and the systemInfoModification bit of Short Message is set:

2>	apply the SI acquisition procedure as defined in clause 5.2.2.3 from the start of the next modification period;

1>	if the UE operates an IDLE eDRX cycle longer than the modification period and the systemInfoModification-eDRX bit of Short Message is set:

2>	apply the SI acquisition procedure as defined in clause 5.2.2.3 from the start of the next eDRX acquisition period boundary.

[TS 38.331, clause 5.2.2.3.1]

The UE shall:

1>	apply the specified BCCH configuration defined in 9.1.1.1;

1>	if the UE is in RRC_IDLE or in RRC_INACTIVE; or

1>	if the UE is in RRC_CONNECTED while T311 is running:

2>	acquire the MIB, which is scheduled as specified in TS 38.213 [13];

...

1>	if the UE is in RRC_IDLE or in RRC_INACTIVE; or

…

2>	if ssb-SubcarrierOffset indicates SIB1 is transmitted in the cell (TS 38.213 [13]) and if SIB1 acquisition is required for the UE:

3>	acquire the SIB1, which is scheduled as specified in TS 38.213 [13];

3>	if the UE is unable to acquire the SIB1:

4>	perform the actions as specified in clause 5.2.2.5;

3>	else:

4>	upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2.

2>	else if SIB1 acquisition is required for the UE and ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell:

3>	perform the actions as specified in clause 5.2.2.5.

…

[TS 38.331, clause 5.2.2.3.2]

For SI message acquisition PDCCH monitoring occasion(s) are determined according to searchSpaceOtherSystemInformation. If searchSpaceOtherSystemInformation is set to zero, PDCCH monitoring occasions for SI message reception in SI-window are same as PDCCH monitoring occasions for SIB1 where the mapping between PDCCH monitoring occasions and SSBs is specified in TS 38.213[13]. If searchSpaceOtherSystemInformation is not set to zero, PDCCH monitoring occasions for SI message are determined based on search space indicated by searchSpaceOtherSystemInformation. PDCCH monitoring occasions for SI message which are not overlapping with UL symbols (determined according to tdd-UL-DL-ConfigurationCommon) are sequentially numbered from one in the SI window. The [x×N+K]th PDCCH monitoring occasion (s) for SI message in SI-window corresponds to the Kth transmitted SSB, where x = 0, 1, ...X-1, K = 1, 2, …N, N is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 and X is equal to CEIL(number of PDCCH monitoring occasions in SI-window/N). The actual transmitted SSBs are sequentially numbered from one in ascending order of their SSB indexes. The UE assumes that, in the SI window, PDCCH for an SI message is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB and thus the selection of SSB for the reception SI messages is up to UE implementation.

When acquiring an SI message, the UE shall:

1>	determine the start of the SI-window for the concerned SI message as follows:

2>	if the concerned SI message is configured in the schedulingInfoList:

3>	for the concerned SI message, determine the number n which corresponds to the order of entry in the list of SI messages configured by schedulingInfoList in si-SchedulingInfo in SIB1;

3>	determine the integer value x = (n – 1) × w, where w is the si-WindowLength;

3>	the SI-window starts at the slot #a, where a = x mod N, in the radio frame for which SFN mod T = FLOOR(x/N), where T is the si-Periodicity of the concerned SI message and N is the number of slots in a radio frame as specified in TS 38.213 [13];

2>	else if the concerned SI message is configured in the schedulingInfoList2;

3>	determine the integer value x = (si-WindowPosition -1) × w, where w is the si-WindowLength;

3>	the SI-window starts at the slot #a, where a = x mod N, in the radio frame for which SFN mod T = FLOOR(x/N), where T is the si-Periodicity of the concerned SI message and N is the number of slots in a radio frame as specified in TS 38.213 [13];

2>	else if the concerned SI message is configured in the posSchedulingInfoList and offsetToSI-Used is not configured:

3>	create a concatenated list of SI messages by appending the posSchedulingInfoList in posSI-SchedulingInfo in SIB1 to schedulingInfoList in si-SchedulingInfo in SIB1;

3>	for the concerned SI message, determine the number n which corresponds to the order of entry in the concatenated list;

3>	determine the integer value x = (n – 1) × w, where w is the si-WindowLength;

3>	the SI-window starts at the slot #a, where a = x mod N, in the radio frame for which SFN mod T = FLOOR(x/N), where T is the posSI-Periodicity of the concerned SI message and N is the number of slots in a radio frame as specified in TS 38.213 [13];

2>	else if the concerned SI message is configured by the posSchedulingInfoList and offsetToSI-Used is configured:

3>	determine the number m which corresponds to the number of SI messages with an associated si-Periodicity of 8 radio frames (80 ms), configured by schedulingInfoList in SIB1;

3>	for the concerned SI message, determine the number n which corresponds to the order of entry in the list of SI messages configured by posSchedulingInfoList in SIB1;

3>	determine the integer value x = m × w + (n – 1) × w, where w is the si-WindowLength;

3>	the SI-window starts at the slot #a, where a = x mod N, in the radio frame for which SFN mod T = FLOOR(x/N) +8, where T is the posSI-Periodicity of the concerned SI message and N is the number of slots in a radio frame as specified in TS 38.213 [13];

1>	receive the PDCCH containing the scheduling RNTI, i.e. SI-RNTI in the PDCCH monitoring occasion(s) for SI message acquisition, from the start of the SI-window and continue until the end of the SI-window whose absolute length in time is given by si-WindowLength, or until the SI message was received;

1>	if the SI message was not received by the end of the SI-window, repeat reception at the next SI-window occasion for the concerned SI message in the current modification period;

1>	if all the SIB(s) and/or posSIB(s) requested in DedicatedSIBRequest message have been acquired:

2>	stop timer T350, if running;

NOTE 1:	The UE is only required to acquire broadcasted SI message if the UE can acquire it without disrupting unicast or MBS multicast data reception, i.e. the broadcast and unicast/MBS multicast beams are quasi co-located.

NOTE 2:	The UE is not required to monitor PDCCH monitoring occasion(s) corresponding to each transmitted SSB in SI-window.

NOTE 3:	If the concerned SI message was not received in the current modification period, handling of SI message acquisition is left to UE implementation.

NOTE 4:	A UE in RRC_CONNECTED may stop the PDCCH monitoring during the SI window for the concerned SI message when the requested SIB(s) are acquired.

NOTE 5:	A UE capable of NR sidelink communication/discovery and configured by upper layers to perform NR sidelink communication/discovery on a frequency, may acquire SIB12 or SystemInformationBlockType28 from a cell other than current serving cell (for RRC_INACTIVE or RRC_IDLE) or current PCell (for RRC_CONNECTED), if SIB12 of current serving cell (for RRC_INACTIVE or RRC_IDLE) or current PCell (for RRC_CONNECTED) does not provide configuration for NR sidelink communication/discovery for the frequency, and if the other cell providing configuration for NR sidelink communication/discovery for the frequency meets the S-criteria as defined in TS 38.304 [20] or TS 36.304 [27].

1>	perform the actions for the acquired SI message as specified in clause 5.2.2.4.

[TS 38.331, clause 5.2.2.4.2]

Upon receiving the SIB1 the UE shall:

1>	store the acquired SIB1;

…

2>	if the UE has a stored valid version of a SIB or posSIB, in accordance with clause 5.2.2.2.1, that the UE requires to operate within the cell in accordance with clause 5.2.2.1:

3>	use the stored version of the required SIB or posSIB;

2>	else:

3>	acquire the required SIB or posSIB requested by upper layer as defined in clause 5.2.2.3.5;

…

4>	if the UE has a stored valid version of a SIB, in accordance with clause 5.2.2.2.1, that the UE requires to operate within the cell in accordance with clause 5.2.2.1:

5>	use the stored version of the required SIB;

4>	if the UE has not stored a valid version of a SIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s), in accordance with clause 5.2.2.1:

5>	for the SI message(s) that, according to the si-SchedulingInfo, contain at least one required SIB and for which si-BroadcastStatus is set to broadcasting:

6>	acquire the SI message(s) as defined in clause 5.2.2.3.2;

5>	for the SI message(s) that, according to the si-SchedulingInfo, contain at least one required SIB and for which si-BroadcastStatus is set to notBroadcasting:

6>	trigger a request to acquire the SI message(s) as defined in clause 5.2.2.3.3;

…

[TS 38.331, clause 6.5]

Short Messages can be transmitted on PDCCH using P-RNTI with or without associated Paging message using Short Message field in DCI format 1_0 (see TS 38.212 [17], clause 7.3.1.2.1).

Table 6.5-1 defines Short Messages. Bit 1 is the most significant bit.

Table 6.5-1: Short Messages

| Bit | Short Message |
| --- | --- |
| 1 | systemInfoModification If set to 1: indication of a BCCH modification other than SIB6, SIB7, SIB8 and posSIBs. |
| 2 | etwsAndCmasIndication If set to 1: indication of an ETWS primary notification and/or an ETWS secondary notification and/or a CMAS notification. |
| 3 | stopPagingMonitoring This bit can be used for only operation with shared spectrum channel access and if nrofPDCCH-MonitoringOccasionPerSSB-InPO is present. If set to 1: indication that the UE may stop monitoring PDCCH occasion(s) for paging in this Paging Occasion as specified in TS 38.304 [20], clause 7.1. |
| 4 | systemInfoModification-eDRX If set to 1: indication of a BCCH modification other than SIB6, SIB7, SIB8 and posSIBs. This indication applies only to UEs using IDLE eDRX cycle longer than the BCCH modification period. |
| 5 – 8 | Not used in this release of the specification and shall be ignored by UE if received. |

8.1.5.2.3.3	Test Description

8.1.5.2.3.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used in NR cell.

UE:

-	UE supporting extended DRX.

Preamble:

-	UE is in state 0N-B according to TS 38.508-1 [4] Table 4.4A.2-0.

8.1.5.2.3.3.2	Test procedure sequence

Table 8.1.5.2.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on | - | - | - | - |
| 2-3 | Steps 2-3 of the generic procedure for NR RRC_IDLE as specified in TS 38.508-1 [4] Table 4.5.2.2-2 are performed. | - | - | - | - |
| 4 | The UE transmits a REGISTRATION REQUEST message including the Requested extended DRX parameters IE. | --> | NR RRC: RRCSetupComplete 5GMM: REGISTRATION REQUEST | - | - |
| 5-13 | Steps 5-13 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 14 | The SS transmits a REGISTRATION ACCEPT message including the Negotiated extended DRX parameters IE. | <-- | NR RRC: DLInformationTransfer 5GMM: REGISTRATION ACCEPT | - | - |
| 15 | The UE transmit a REGISTRATION COMPLETE message | --> | NR RRC: ULInformationTransfer 5GMM: REGISTRATION COMPLETE | - | - |
| 16-17 | Steps 19Ca1-19Ca2 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 18 | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| 19 | The SIB1 is updated according to Table 8.1.5.2.3.3.3-2 and SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification-eDRX. | <-- | PDCCH (DCI 1_0): Short Message | - | - |
| 20 | Wait till the beginning of next eDRX acquisition time (next H-SFN for which H-SFN mod 1024 =0). | - | - | - | - |
| 20A | After Step 20, wait for 10s for the UE to acquire the new system information. | - | - | - | - |
| 21 | The SS transmits a Paging message within a paging occasion | <-- | NR RRC: Paging | - | - |
| 22 | Check: Does the UE transmit a random access using prach-ConfigurationIndex given in step 19? | - | - | 1 | P |
| 23 | The UE transmits an RRCSetupRequest | --> | NR RRC: RRCSetupRequest | - | - |
| 24-29 | Steps 3 to 8 of the NR RRC_CONNECTED procedure in TS 38.508-1 Table 4.5.4.2-3 are executed | - | - | - | - |
| 30 | Switch off UE in RRC CONNECTED as described in TS 38.508-1 [4] subclause 4.9.6.3 | - |  |  |  |
| 31 | Switch on UE. | - |  |  |  |
| 32-33 | Steps 2-3 of the generic procedure for NR RRC_IDLE as specified in TS 38.508-1 [4] Table 4.5.2.2-2 are performed. | - | - | - | - |
| 34 | The UE transmits a REGISTRATION REQUEST message including the Requested extended DRX parameters IE. | --> | NR RRC: RRCSetupComplete 5GMM: REGISTRATION REQUEST | - | - |
| 35-43 | Steps 5-13 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 44 | The SS transmits a REGISTRATION ACCEPT message including the Negotiated extended DRX parameters IE. | <-- | NR RRC: DLInformationTransfer 5GMM: REGISTRATION ACCEPT | - | - |
| 45 | The UE transmit a REGISTRATION COMPLETE message | --> | NR RRC: ULInformationTransfer 5GMM: REGISTRATION COMPLETE | - | - |
| 46-47 | Steps 19a1-19a2a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 48 | IF connected without release is not present THEN, the SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| 49 | The SIB1 is updated according to Table 8.1.5.2.3.3.3-1 and SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. | <-- | PDCCH (DCI 1_0): Short Message | - | - |
| 50 | Wait for 2.1* modification period to allow the new system information to take effect. | - | - | - | - |
| 51 | The SS transmits a Paging message within a paging occasion | <-- | NR RRC: Paging | - | - |
| 52 | Check: Does the UE transmit a random access using prach-ConfigurationIndex given in step 49? | - | - | 2 | P |
| 53 | The UE transmits an RRCSetupRequest | --> | NR RRC: RRCSetupRequest | - | - |
| 54-59 | Steps 3 to 8 of the NR RRC_CONNECTED procedure in TS 38.508-1 Table 4.5.4.2-3 are executed to successfully complete the service request procedure. | - | - | - | - |

8.1.5.2.3.3.3	Specific message contents

Table 8.1.5.2.3.3.3-1: SIB1 (Preamble and Step49, Table 8.1.5.2.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| hyperSFN-r17 | Current H-SFN Value |  |  |
| eDRX-AllowedIdle-r17 | true |  |  |
| eDRX-AllowedInactive-r17 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.2.3.3.3-2: SIB1 (Step19, Table 8.1.5.2.3.3.2-1)

| Derivation Path: TS 38.508-1 [4] table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| hyperSFN-r17 | Current H-SFN Value |  |  |
| eDRX-AllowedIdle-r17 | true |  |  |
| eDRX-AllowedInactive-r17 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| servingCellConfigCommon SEQUENCE { |  |  |  |
| uplinkConfigCommon SEQUENCE { |  |  |  |
| initialUplinkBWP SEQUENCE { |  |  |  |
| rach-ConfigCommon SEQUENCE { |  |  |  |
| rach-ConfigGeneric SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 119 |  | FR1 FDD |
|  | 120 |  | FR1 FDD AND HD_FDD |
|  | 91 |  | FR1 TDD |
|  | 6 |  | FR2 |
| ra-ResponseWindow | sl20 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| HD_FDD | pc_halfDuplexFDD_TypeA_RedCap_r17 (i.e HD_FDD UE are performing test on FDD band) |

Table 8.1.5.2.3.3.3-3: REGISTRATION ACCEPT (Step 14, Table 8.1.5.2.3.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Negotiated extended DRX parameters |  |  |  |
| Paging Time Window | Any allowed value | Note |  |
| eDRX value | ‘0100’B | 40,96 seconds |  |
| Extended Paging Time Window | ‘00000001’B | 2,56 seconds |  |
| Note 1: In NR connected to 5GCN, the Paging Time Window field is ignored and the PTW value is included in the Extended Paging Time Window field. |  |  |  |

Table 8.1.5.2.3.3.3-4: REGISTRATION ACCEPT (Step 44, Table 8.1.5.2.3.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Negotiated extended DRX parameters |  |  |  |
| Paging Time Window | Any allowed value | Note 1 |  |
| eDRX value | ‘0000’B | 2,56 seconds |  |
| Extended Paging Time Window | ‘00000011’B | 5,12 seconds |  |
| Note 1: In NR connected to 5GCN, the Paging Time Window field is ignored and the PTW value is included in the Extended Paging Time Window field. |  |  |  |

Table 8.1.5.2.3.3.3-5: REGISTRATION REQUEST (Step 4 and Step 52 Table 8.1.5.2.3.3.2-1)

| Derivation path: TS 38.508-1 [4], table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Requested extended DRX parameters |  |  |  |
| Paging Time Window | Present but contents not checked |  |  |
| eDRX value | Present but contents not checked |  |  |
| Extended Paging Time Window | Not Checked | Note 1 |  |
| Note 1: Extended Paging Time Window is an optional IE. If UE request eDRX cycle larger than 10,24 seconds, UE may include Extended Paging Time Window. |  |  |  |

#### 8.1.5.3	PWS notification

##### 8.1.5.3.1	PWS notification / PWS reception in NR RRC_IDLE state

8.1.5.3.1.1	Test Purpose (TP)

(1)

with { UE in NR RRC_IDLE state }

ensure that {

when { an ETWS capable UE receives a short message transmitted on PDCCH using P-RNTI indicating an etwsAndCmasIndication and the si-SchedulingInfo includes scheduling information for SIB6 }

then { the UE is able to retrieve the PWS message from SIB6 and alert the user }

}

(2)

with { UE in NR RRC_IDLE state }

ensure that {

when { an ETWS capable UE receives a short message transmitted on PDCCH using P-RNTI indicating an etwsAndCmasIndication and the si-SchedulingInfo includes scheduling information for SIB7 }

then { the UE is able to retrieve all the PWS message segments from SIB7, reassemble the message and alert the user }

}

(3)

with { UE in NR RRC_IDLE state }

ensure that {

when { a CMAS capable UE receives a short message transmitted on PDCCH using P-RNTI indicating an etwsAndCmasIndication and the si-SchedulingInfo includes scheduling information for SIB8 }

then { the UE is able to retrieve all the PWS message segments from SIB8, reassemble the message and alert the user }

}

8.1.5.3.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331 clause 5.2.2.2.2.

[TS 38.331 clause 5.2.2.2.2]

…

ETWS or CMAS capable UEs in RRC_IDLE or in RRC_INACTIVE shall monitor for indications about PWS notification in its own paging occasion every DRX cycle. ETWS or CMAS capable UEs in RRC_CONNECTED shall monitor for indication about PWS notification in any paging occasion at least once every defaultPagingCycle if the UE is provided with common search space on the active BWP to monitor paging.

For Short Message reception in a paging occasion, the UE monitors the PDCCH monitoring occasion(s) for paging as specified in TS 38.304 [20] and TS 38.213 [13].

If the UE receives a Short Message, the UE shall:

1>	if the UE is ETWS capable or CMAS capable, the etwsAndCmasIndication bit of Short Message is set, and the UE is provided with searchSpaceOtherSystemInformation on the active BWP:

2> immediately re-acquire the SIB1;

2>	if the UE is ETWS capable and si-SchedulingInfo includes scheduling information for SIB6:

3>	acquire SIB6, as specified in sub-clause 5.2.2.3.2, immediately;

2>	if the UE is ETWS capable and si-SchedulingInfo includes scheduling information for SIB7:

3>	acquire SIB7, as specified in sub-clause 5.2.2.3.2, immediately;

2>	if the UE is CMAS capable and si-SchedulingInfo includes scheduling information for SIB8:

3>	acquire SIB8, as specified in sub-clause 5.2.2.3.2, immediately;

1> if the systemInfoModification bit of Short Message is set:

2>	apply the SI acquisition procedure as defined in sub-clause 5.2.2.3 from the start of the next modification period.

8.1.5.3.1.3	Test description

8.1.5.3.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

UE:

-	None.

Preamble:

-	The UE is in test state 1N-A according to TS 38.508-1 [4].

8.1.5.3.1.3.2	Test procedure sequence

Table 8.1.5.3.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | If pc_ETWS_NR is true, step1-8 will be excuted. | - | - | - | - |
| 1 | The SS indicates an etwsAndCmasIndication by Short Message field in DCI format 1_0 and transmitted on PDCCH using P-RNTI. The SS starts transmitting SIB1 with the si-SchedulingInfo including scheduling information for SIB6 on NR Cell 1. | <-- | PDCCH (DCI 1_0): Short Message | - | - |
| 2 | The SS includes an ETWS message with new messageIdentifier and serialNumber in SIB6. (Note 1) | - | - | - | - |
| 3 | Check: Does the UE alert or activate alerting the user (NOTE 2)? | - | - | 1 | P |
| 4 | The SS waits for 10s. | - | - | - | - |
| 5 | The SS indicates an etwsAndCmasIndication by Short Message field in DCI format 1_0 and transmitted on PDCCH using P-RNTI. The SS starts transmitting SIB1 with the si-SchedulingInfo including scheduling information for SIB7 on NR Cell 1. | <-- | PDCCH (DCI 1_0): Short Message | - | - |
| 6 | The SS includes an ETWS message with new messageIdentifier and serialNumber in SIB7. (NOTE 3). | - | - | - | - |
| 7 | Check: Does the UE indicate the contents of the "warning message" to the user, and alert or activate alerting the user (NOTE 2)? | - | - | 2 | P |
| 8 | The SS waits for 10s. | - | - | - | - |
| - | If pc_CMAS_NR is true, step9-11 will be excuted. | - | - | - | - |
| 9 | The SS indicates an etwsAndCmasIndication by Short Message field in DCI format 1_0 and transmitted on PDCCH using P-RNTI. The SS starts transmitting SIB1 with the si-SchedulingInfo including scheduling information for SIB8 on NR Cell 1. | <-- | PDCCH (DCI 1_0): Short Message | - | - |
| 10 | The SS includes a CMAS message with new messageIdentifier and serialNumber in SIB8. (NOTE 4). | - | - | - | - |
| 11 | Check: Does the UE indicate the contents of the "warning message" to the user, and alert or activate alerting the user (NOTE 2)? | - | - | 3 | P |
| NOTE 1: The NR-10 in Table 4.4.3.1.2-1 Combinations of system information blocks of TS 38.508-1 [4] is used. NOTE 2: The data indication and user alerting are the UE implementation issues. NOTE 3: The NR-11 in Table 4.4.3.1.2-1 Combinations of system information blocks of TS 38.508-1 [4] is used. NOTE 4: The NR-9 in Table 4.4.3.1.2-1 Combinations of system information blocks of TS 38.508-1 [4] is used. |  |  |  |  |  |

8.1.5.3.1.3.3	Specific message contents

None

##### 8.1.5.3.2	PWS notification / PWS reception in NR RRC_INACTIVE state

8.1.5.3.2.1	Test Purpose (TP)

(1)

with { UE in NR RRC_INACTIVE state }

ensure that {

when { an ETWS capable UE receives a short message transmitted on PDCCH using P-RNTI indicating an etwsAndCmasIndication and the si-SchedulingInfo includes scheduling information for SIB6 }

then { the UE is able to retrieve the PWS message from SIB6 and alert the user }

}

(2)

with { UE in NR RRC_INACTIVE state }

ensure that {

when { an ETWS capable UE receives a short message transmitted on PDCCH using P-RNTI indicating an etwsAndCmasIndication and the si-SchedulingInfo includes scheduling information for SIB7 }

then { the UE is able to retrieve all the PWS message segments from SIB7, reassemble the message and alert the user }

}

(3)

with { UE in NR RRC_INACTIVE state }

ensure that {

when { a CMAS capable UE receives a short message transmitted on PDCCH using P-RNTI indicating an etwsAndCmasIndication and the si-SchedulingInfo includes scheduling information for SIB8 }

then { the UE is able to retrieve all the PWS message segments from SIB8, reassemble the message and alert the user }

}

8.1.5.3.2.2	Conformance requirements

Same as test case 8.1.5.3.1.

8.1.5.3.2.3	Test description

8.1.5.3.2.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

UE:

None.

Preamble:

-	The UE is in state Registered, Inactive mode (state 2N-A) according to TS 38.508-1 [4].

8.1.5.3.2.3.2	Test procedure sequence

Same as test case 8.1.5.3.1

8.1.5.3.2.3.3	Specific message contents

Same as test case 8.1.5.3.1.

##### 8.1.5.3.3	PWS notification / PWS reception in NR RRC_CONNECTED state

8.1.5.3.3.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { an ETWS capable UE receives a short message transmitted on PDCCH using P-RNTI indicating an etwsAndCmasIndication and the si-SchedulingInfo includes scheduling information for SIB6 }

then { the UE is able to retrieve all the PWS message segments from SIB6, reassemble the message and alert the user }

}

(2)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { an ETWS capable UE receives a short message transmitted on PDCCH using P-RNTI indicating an etwsAndCmasIndication and the si-SchedulingInfo includes scheduling information for SIB7 }

then { the UE is able to retrieve all the PWS message segments from SIB7, reassemble the message and alert the user }

}

(3)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { a CMAS capable UE receives a short message transmitted on PDCCH using P-RNTI indicating an etwsAndCmasIndication and the si-SchedulingInfo includes scheduling information for SIB8 }

then { the UE is able to retrieve all the PWS message segments from SIB8, reassemble the message and alert the user }

}

8.1.5.3.3.2	Conformance requirements

Same as test case 8.1.5.3.1.

8.1.5.3.3.3	Test description

8.1.5.3.3.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

UE:

None.

Preamble:

-	The UE is in test state 3N-A according to TS 38.508-1 [4].

8.1.5.3.3.3.2	Test procedure sequence

Same as test case 8.1.5.3.1 except PDCCH (DCI 1_0): Short Message is sent on active BWP.

8.1.5.3.3.3.3	Specific message contents

Same as test case 8.1.5.3.1.

##### 8.1.5.3.4	PWS notification / PWS reception using dedicatedSystemInformationDelivery

8.1.5.3.4.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE receives an RRCReconfiguration message including dedicatedSystemInformationDelivery containing SIB6 }

then { UE reads the SIB6 to alert the user and sends an RRCReconfigurationComplete message }

}

(2)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE receives an RRCReconfiguration message including dedicatedSystemInformationDelivery containing SIB7 }

then { UE reads the SIB7 to alert the user and sends an RRCReconfigurationComplete message }

}

(3)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE receives an RRCReconfiguration message including dedicatedSystemInformationDelivery containing SIB8 }

then { UE reads the SIB8 to alert the user and sends an RRCReconfigurationComplete message }

}

8.1.5.3.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331, clauses 5.2.2.4.7, 5.2.2.4.8, 5.2.2.4.9,  5.3.5.3. Unless otherwise stated these are Rel-15 requirements.

[TS 38.331, clause 5.3.5.3]

The UE shall perform the following actions upon reception of the RRCReconfiguration:

1>	if the RRCReconfiguration message includes the dedicatedSIB1-Delivery:

2>	perform the action upon reception of SIB1 as specified in 5.2.2.4.2;

1>	if the RRCReconfiguration message includes the dedicatedSystemInformationDelivery:

2>	perform the action upon reception of System Information as specified in 5.2.2.4;

[TS 38.331, clause 5.2.2.4.7]

Upon receiving the SIB6 the UE shall:

1>	forward the received warningType, messageIdentifier and serialNumber to upper layers;

[TS 38.331, clause 5.2.2.4.8]

Upon receiving the SIB7 the UE shall:

1>	if there is no current value for messageIdentifier and serialNumber for SIB7; or

1>	if either the received value of messageIdentifier or of serialNumber or of both are different from the current values of messageIdentifier and serialNumber for SIB7:

2>	use the received values of messageIdentifier and serialNumber for SIB7 as the current values of messageIdentifier and serialNumber for SIB7;

2>	discard any previously buffered warningMessageSegment;

2>	if all segments of a warning message have been received:

3>	assemble the warning message from the received warningMessageSegment;

3>	forward the received warning message, messageIdentifier, serialNumber and dataCodingScheme to upper layers;

3>	stop reception of SIB7;

3>	discard the current values of messageIdentifier and serialNumber for SIB7;

2>	else:

3>	store the received warningMessageSegment;

3>	continue reception of SIB7;

1>	else if all segments of a warning message have been received:

2>	assemble the warning message from the received warningMessageSegment;

2>	forward the received complete warning message, messageIdentifier, serialNumber and dataCodingScheme to upper layers;

2>	stop reception of SIB7;

2>	discard the current values of messageIdentifier and serialNumber for SIB7;

1>	else:

2>	store the received warningMessageSegment;

2>	continue reception of SIB7;

The UE should discard any stored warningMessageSegment and the current value of messageIdentifier and serialNumber for SIB7 if the complete warning message has not been assembled within a period of 3 hours.

[TS 38.331, clause 5.2.2.4.9]

Upon receiving the SIB8 the UE shall:

1>	if the SIB8 contains a complete warning message and the complete geographical area coordinates (if any):

2>	forward the received warning message, messageIdentifier, serialNumber, dataCodingScheme and the geographical area coordinates (if any) to upper layers;

2>	continue reception of SIB8;

1>	else:

2>	if the received values of messageIdentifier and serialNumber are the same (each value is the same) as a pair for which a warning message and the geographical area coordinates (if any) are currently being assembled:

3>	store the received warningMessageSegment;

3>	store the received warningAreaCoordinatesSegment (if any);

3>	if all segments of a warning message and geographical area coordinates (if any) have been received:

4>	assemble the warning message from the received warningMessageSegment;

4>	assemble the geographical area coordinates from the received warningAreaCoordinatesSegment (if any);

4>	forward the received warning message, messageIdentifier, serialNumber, dataCodingScheme and geographical area coordinates (if any) to upper layers;

4>	stop assembling a warning message and geographical area coordinates (if any) for this messageIdentifier and serialNumber and delete all stored information held for it;

3>	continue reception of SIB8;

2>	else if the received values of messageIdentifier and/or serialNumber are not the same as any of the pairs for which a warning message is currently being assembled:

3>	start assembling a warning message for this messageIdentifier and serialNumber pair;

3>	start assembling the geographical area coordinates (if any) for this messageIdentifier and serialNumber pair;

3>	store the received warningMessageSegment;

3>	store the received warningAreaCoordinatesSegment (if any);

3>	continue reception of SIB8;

The UE should discard warningMessageSegment and warningAreaCoordinatesSegment (if any) and the associated values of messageIdentifier and serialNumber for SIB8 if the complete warning message and the geographical area coordinates (if any) have not been assembled within a period of 3 hours.

NOTE:	The number of warning messages that a UE can re-assemble simultaneously is a function of UE implementation.

8.1.5.3.4.3	Test description

8.1.5.3.4.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

UE:

-	None

Preamble:

-	The UE is in 5GS state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-1

8.1.5.3.4.3.2	Test procedure sequence

Table 8.1.5.3.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | If pc_ETWS_NR is true, steps 1-6 will be excuted. | - | - | - | - |
| 1 | The SS transmits an RRCReconfiguration message containing dedicatedSystemInformationDelivery containing SIB6. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | Check: Does the UE transmit an RRCReconfigurationComplete message? | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | Check: Does the UE alert or activate alerting the user (NOTE 1)? | - | - | 1 | P |
| 4 | The SS transmits an RRCReconfiguration message containing dedicatedSystemInformationDelivery containing SIB7. | <-- | NR RRC: RRCReconfiguration | - | - |
| 5 | Check: Does the UE transmit an RRCReconfigurationComplete message? | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 6 | Check: Does the UE indicate the "warning message" to the user, and alert or activate alerting the user (NOTE 1)? | - | - | 2 | P |
| - | If pc_CMAS_NR is true, steps 7-9 will be excuted. | - | - | - | - |
| 7 | The SS transmits an RRCReconfiguration message containing dedicatedSystemInformationDelivery containing SIB8. | <-- | NR RRC: RRCReconfiguration | - | - |
| 8 | Check: Does the UE transmit an RRCReconfigurationComplete message? | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 9 | Check: Does the UE alert or activate alerting the user (NOTE 1)? | - | - | 3 | P |
| NOTE 1: The data indication and user alerting are the UE implementation issues. |  |  |  |  |  |

8.1.5.3.4.3.3	Specific message contents

Table 8.1.5.3.4.3.3-1: RRCReconfiguration (Steps 1, 4 and 7 Table 8.1.5.3.4.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | Not present |  |  |
| dedicatedNAS-MessageList | Not present |  |  |
| dedicatedSystemInformationDelivery | SystemInformation | OCTET STRING (CONTAINING SystemInformation) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.3.4.3.3-2: SystemInformation (Steps 1, 4 and 7 Table 8.1.5.3.4.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-29 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| systemInformation-r15 SEQUENCE { |  |  |  |
| sib-TypeAndInfo SEQUENCE (SIZE (1..maxSIB)) OF CHOICE { | 1 entry |  |  |
| sib6 | SIB6 | Acc to 38.508-1 [4] Table 4.6.2-5: SIB6 | Step 1 |
| sib7 | SIB7 |  | Step 4 |
| sib8 | SIB8 |  | Step 7 |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.3.4.3.3-3: SIB7 (Step 4 in Table 8.1.5.3.4.3.2-1)

| Derivation Path: TS 38.331 [6], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB7 ::= SEQUENCE { |  |  |  |
| messageIdentifier | '0001 0001 0000 0010'B | ETWS message identifier for earthquake and tsunami message (see TS 23.041 [25]) |  |
| serialNumber | '0011 0000 0000 0000'B | Note 1. |  |
| warningMessageSegmentType | lastSegment |  |  |
| warningMessageSegmentNumber | 0 |  |  |
| warningMessageSegment | Octetstring of N | Containing the complete ETWS message |  |
| dataCodingScheme | Bitstring (8) ID of the alphabet/coding and the applied language | see TS 23.041 [25]. |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |
| Note 1: Geographical Scope (Octet 1 bit 7 ~ 6) set to 'Cell wide', Emergency User Alert (Octet 1 bit 5) set to 'Activate emergency user alert', Popup (Octet 1 bit 4) set to 'Activate popup', Update Number (Octet 2 bits 3~0) for each update, incremented by one, See TS 23.041 [25]. |  |  |  |

Table 8.1.5.3.4.3.3-4: SIB8 (Step 7 in Table 8.1.5.3.4.3.2-1)

| Derivation Path: TS 38.331 [6], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB8 ::= SEQUENCE { |  |  |  |
| messageIdentifier | '0001 0001 0001 0010'B | CMAS CBS Message Identifier for CMAS Presidential Level Alerts (see TS 23.041 [25]) |  |
| serialNumber | '0011 0000 0000 0000'B | Note 1 |  |
| warningMessageSegmentType | lastSegment |  |  |
| warningMessageSegmentNumber | 0 |  |  |
| warningMessageSegment | Octetstring of N |  |  |
| dataCodingScheme | Bitstring (8) ID of the alphabet/coding and the applied language | Containing the complete CMAS message |  |
| warningAreaCoordinatesSegment | Not present |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |
| Note 1: Geographical Scope (Octet 1 bit 7 ~ 6) set to 'Cell wide', Emergency User Alert (Octet 1 bit 5) set to 'Activate emergency user alert', Popup (Octet 1 bit 4) set to 'Activate popup', Update Number (Octet 2 bits 3~0) for each update, incremented by one, See TS 23.041 [25]. |  |  |  |

#### 8.1.5.4	Counter check

##### 8.1.5.4.1	Counter check / Reception of CounterCheck message by the UE

8.1.5.4.1.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state with two DRBs established and having received 10 data packets on each DRB }

ensure that {

when { UE receives CounterCheck message with drb-CountMSB-InfoList included for DRB3(not established) and DRB1 (with the most significant bits of the count different from the actual COUNT)}

then { UE sends CounterCheckResponse message by including the drb-Identity, the count-Uplink and the count-Downlink set to the value of TX_NEXT – 1 and RX_NEXT – 1 in drb-CountInfoList IE for each DRB established, and by including the drb-Identity, the count-Uplink and the count-Downlink with the most significant bits set identical to the corresponding values in the drb-CountMSB-InfoList and the least significant bits set to zero for the DRB that is not established }

}

8.1.5.4.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clause 5.3.6.3. Unless otherwise stated these are Rel-15 requirements.

[TS 38.331, clause 5.3.6.3]

Upon receiving the CounterCheck message, the UE shall:

1>	for each DRB that is established:

2>	if no COUNT exists for a given direction (uplink or downlink) because it is a uni-directional bearer configured only for the other direction:

3>	assume the COUNT value to be 0 for the unused direction;

2>	if the drb-Identity is not included in the drb-CountMSB-InfoList:

3>	include the DRB in the drb-CountInfoList in the CounterCheckResponse message by including the drb-Identity, the count-Uplink and the count-Downlink set to the value of TX_NEXT – 1 and RX_NEXT – 1 (specified in TS 38.323 [5]), respectively;

2>	else if, for at least one direction, the most significant bits of the COUNT are different from the value indicated in the drb-CountMSB-InfoList:

3>	include the DRB in the drb-CountInfoList in the CounterCheckResponse message by including the drb-Identity, the count-Uplink and the count-Downlink set to the value of TX_NEXT – 1 and RX_NEXT – 1 (specified in TS 38.323 [5]), respectively;

1>	for each DRB that is included in the drb-CountMSB-InfoList in the CounterCheck message that is not established:

2>	include the DRB in the drb-CountInfoList in the CounterCheckResponse message by including the drb-Identity, the count-Uplink and the count-Downlink with the most significant bits set identical to the corresponding values in the drb-CountMSB-InfoList and the least significant bits set to zero;

1>	submit the CounterCheckResponse message to lower layers for transmission upon which the procedure ends.

8.1.5.4.1.3	Test description

8.1.5.4.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used in  the NR cell.

UE:

-	None.

Preamble:

-	The UE is in 5GS state 3N-A on NR Cell 1 with atleast one PDU session active according to TS 38.508-1 [4], clause 4.4A.3 Table 4.4A.3-1 and Test Loop Function (On) with UE test loop mode B is established. 2 DRBs are configured where DRB1 is defined as default DRB.

8.1.5.4.1.3.2	Test procedure sequence

Table 8.1.5.4.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits 10 IP packets on each DRB (using QFI2 for DRB1 and QFI5 for DRB2) | - | - | - | - |
| 2 | The UE loops back the IP packets received in step 1 (Note 1) | - | - | - | - |
| 3 | The SS transmits CounterCheck message with drb-Identity not included in the drb-CountMSB-InfoList | <-- | NR RRC: CounterCheck | - | - |
| 4 | Check: Does the UE send CounterCheckResponse message by including the drb-Identity, the count-Uplink and the count-Downlink set to the value of TX_NEXT – 1 and RX_NEXT – 1 in drb-CountInfoList IE for each DRB established? | --> | NR RRC: CounterCheckResponse | 1 | P |
| Note 1: IP packets at step 2 may be received by the SS in the same slot or in multiple slots |  |  |  |  |  |

8.1.5.4.1.3.3	Specific message contents

Table 8.1.5.4.1.3.3-1: RRCReconfiguration (Preamble, Table 8.1.5.4.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], table 4.6.1-13 with condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig-2DRBs |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig-2DRBs | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.4.1.3.3-2: RadioBearerConfig-2DRBs (Preamble, Table 8.1.5.4.1.3.3-1)

| Derivation Path: TS 38.508-1 [4], table 4.6.3-132 and condition SRB2 and DRB1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 2 entries |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config::= SEQUENCE { |  |  |  |
| pdu-Session | PDU session ID | Same as the PDU session ID in PDU SESSION ESTABLISHMENT REQUEST |  |
| defaultDRB | true |  |  |
| mappedQoS-FlowsToAdd SEQUENCE { |  |  |  |
| QFI | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| drb-Identity | DRB-Identity with Condition DRB1 |  |  |
| } |  |  |  |
| } |  |  |  |
| DRB-ToAddMod[2] SEQUENCE |  | entry 2 |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config SEQUENCE { |  |  |  |
| pdu-Session | PDU session ID | Same as the PDU session ID in PDU SESSION ESTABLISHMENT REQUEST |  |
| defaultDRB | false |  |  |
| mappedQoS-FlowsToAdd SEQUENCE { |  |  |  |
| QFI | 5 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| drb-Identity | DRB-Identity with Condition DRB2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.4.1.3.3-3: CellGroupConfig-2DRBs (Preamble, Table 8.1.5.4.1.3.3-1)

| Derivation Path: TS 38.508-1 [4], table 4.6.3-19 and condition SRB2_DRB1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-BearerConfig { | 3 entries |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig with condition SRB2 | entry 1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions AM and DRB1 | entry 2 |  |
| RLC-BearerConfig[3] | RLC-BearerConfig with conditions AM and DRB2 | entry 3 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.4.1.3.3-4: PDU SESSION ESTABLISHMENT ACCEPT (Preamble) (Table 8.1.5.4.1.3.2-1)

| Derivation Path: TS 38.508-1, table 4.7.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDU session ID | The same as the PDU session ID in PDU SESSION ESTABLISHMENT REQUEST |  |  |
| Authorized QoS rules | 3 entries |  |  |
| QoS rule [1] | Reference QoS rule #1 as defined in Table 4.8.2.1-1. | QFI=1 |  |
| QoS rule [2] | Reference QoS rule #4 as defined in Table 4.8.2.1-4 except DQR bit set to ‘0’B. | QFI=2 |  |
| QoS rule [3] | Reference QoS rule #5 as defined in Table 4.8.2.1-5. | QFI=5 |  |
| Mapped EPS Bearer contexts | Not Present |  |  |
| Authorized QoS flow descriptions | 3 entries |  |  |
| QoS flow [1] | Reference QoS flow #1 as defined in Table8.1.5.4.1.3.3-7. | QFI=1 |  |
| QoS flow [2] | Reference QoS flow #2 as defined in Table8.1.5.4.1.3.3-8. | QFI=2 |  |
| QoS flow [3] | Reference QoS flow #3 as defined in Table 4.8.2.3-3. | QFI=5 |  |

Table 8.1.5.4.1.3.3-5: CounterCheck (step 3, Table 8.1.5.4.1.3.2-1)

| Derivation Path: TS 38.508-1 [4] table 4.6.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| CounterCheck ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| counterCheck SEQUENCE { |  |  |  |
| drb-CountMSB-InfoList SEQUENCE (SIZE (1..maxDRB)) OF DRB-CountMSB-Info { | 2 entries |  |  |
| DRB-CountMSB-Info[1] SEQUENCE { |  | entry 1 |  |
| drb-Identity | DRB-Identity with Condition DRB1 | DRB established |  |
| countMSB-Uplink | 1 |  |  |
| countMSB-Downlink | 1 |  |  |
| } |  |  |  |
| DRB-CountMSB-Info[2] SEQUENCE { |  | entry 2 |  |
| drb-Identity | DRB-Identity with Condition DRB3 | DRB not established |  |
| countMSB-Uplink | 3 |  |  |
| countMSB-Downlink | 3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.4.1.3.3-6: CounterCheckResponse (step 4, Table 8.1.5.4.1.3.2-1)

| Derivation Path: TS 38.508-1 [4] table 4.6.1-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| CounterCheckResponse ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| counterCheckResponse SEQUENCE { |  |  |  |
| drb-CountInfoList SEQUENCE (SIZE (0..maxDRB)) OF DRB-CountInfo { | 3 entries |  |  |
| DRB-CountInfo[1] SEQUENCE { |  | entry 1 |  |
| drb-Identity | DRB-Identity with Condition DRB1 | DRB established |  |
| count-Uplink | 9 |  |  |
| count-Downlink | 9 |  |  |
| } |  |  |  |
| DRB-CountInfo[2] SEQUENCE { |  | entry 2 |  |
| drb-Identity | DRB-Identity with Condition DRB2 | DRB established but not requested |  |
| count-Uplink | 9 |  |  |
| count-Downlink | 9 |  |  |
| } |  |  |  |
| DRB-CountInfo[3] SEQUENCE { |  | entry 3 |  |
| drb-Identity | DRB-Identity with Condition DRB3 | DRB not established |  |
| count-Uplink] | 384 | MSB 3 and LSB 0 |  |
| count-Downlink | 384 | MSB 3 and LSB 0 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.4.1.3.3-7: Reference QoS flow #1 (Preamble)

| Derivation Path: TS 38.508-1, table 4.8.2.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QFI | ‘00 0001’B | QFI 1 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| 5QI | ‘0000 1001’B | 5QI 9 |  |

Table 8.1.5.4.1.3.3-8: Reference QoS flow #2 (Preamble)

| Derivation Path: TS 38.508-1, table 4.8.2.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| QoS flow description |  |  |  |
| QFI | ‘00 0010’B | QFI 2 |  |
| Operation code | ‘001’B | Create new QoS flow description |  |
| E bit | ‘1’B | Parameters list is included |  |
| Number of parameters | ’00 0001’B | 1 parameter |  |
| 5QI | ‘0000 0101’B | 5QI 5 |  |

#### 8.1.5.5	Redirection to NR

##### 8.1.5.5.1	Redirection to NR / From E-UTRA / Success

8.1.5.5.1.1	Test Purpose (TP)

(1)

with { UE in  E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives an RRCConnectionRelease message including a redirectedCarrierInfo for redirection to an NR cell }

then { UE enters E-UTRA RRC_IDLE state and performs redirection to NR cell }

}

8.1.5.5.1.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 36.331 clause 5.3.8.3. Unless otherwise stated these are Rel-15 requirements.

[TS 36.331, clause 5.3.8.3]

The UE shall:

1>	except for NB-IoT, BL UEs or UEs in CE, delay the following actions defined in this sub-clause 60 ms from the moment the RRCConnectionRelease message was received or optionally when lower layers indicate that the receipt of the RRCConnectionRelease message has been successfully acknowledged, whichever is earlier;

1>	for BL UEs or UEs in CE, delay the following actions defined in this sub-clause 1.25 seconds from the moment the RRCConnectionRelease message was received or optionally when lower layers indicate that the receipt of the RRCConnectionRelease message has been successfully acknowledged, whichever is earlier;

1>	for NB-IoT, delay the following actions defined in this sub-clause 10 seconds from the moment the RRCConnectionRelease message was received or optionally when lower layers indicate that the receipt of the RRCConnectionRelease message has been successfully acknowledged, whichever is earlier.

NOTE:	For BL UEs, UEs in CE and NB-IoT, when STATUS reporting, as defined in TS 36.322 [7], has not been triggered and the UE has sent positive HARQ feedback (ACK), as defined in TS 36.321 [6], the lower layers can be considered to have indicated that the receipt of the RRCConnectionRelease message has been successfully acknowledged.

1>	stop T380, if running;

1>	if T309 is running:

2>	stop timer T309 for all access categories;

2>	perform the actions as specified in 5.3.16.4.

1>	if the RRCConnectionRelease message is received in response to an RRCConnectionResumeRequest for EDT:

2>	indicate to upper layers that the suspended RRC connection has been resumed;

2>	discard the stored UE AS context and resumeIdentity;

2>	stop timer T300;

2>	stop timer T302, if running;

2>	stop timer T303, if running;

2>	stop timer T305, if running;

2>	stop timer T306, if running;

2>	stop timer T308, if running;

2>	perform the actions as specified in 5.3.3.7;

2>	stop timer T320, if running;

2>	stop timer T322, if running;

1>	if the security is not activated and if UE is connected to 5GC:

2>	perform the actions upon leaving RRC_CONNECTED or RRC_INACTIVE as specified in 5.3.12 with the release cause 'other' upon which the procedure ends;

1>	if the RRCConnectionRelease message includes redirectedCarrierInfo indicating redirection to geran; or

1>	if the RRCConnectionRelease message includes idleModeMobilityControlInfo including freqPriorityListGERAN:

2>	if AS security has not been activated; and

2>	if upper layers indicate that redirect to GERAN without AS security is not allowed:

3>	ignore the content of the RRCConnectionRelease;

3>	perform the actions upon leaving RRC_CONNECTED or RRC_INACTIVE as specified in 5.3.12, with release cause 'other', upon which the procedure ends;

1>	if AS security has not been activated:

2>	ignore the content of redirectedCarrierInfo, if included and indicating redirection to nr;

2>	ignore the content of idleModeMobilityControlInfo, if included and including freqPriorityListNR;

2>	if the UE ignores the content of redirectedCarrierInfo or of idleModeMobilityControlInfo:

3>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other', upon which the procedure ends;

1>	if the RRCConnectionRelease message includes redirectedCarrierInfo indicating redirection to eutra and if UE is connected to 5GC:

2>	if cn-Type is included:

3>	after the cell selection, indicate the available CN Type(s) and the received cn-Type to upper layers;

NOTE 1:	Handling the case if the E-UTRA cell selected after the redirection does not support the core network type specified by the cn-Type, is up to UE implementation.

1>	if the RRCConnectionRelease message includes the idleModeMobilityControlInfo:

2>	store the cell reselection priority information provided by the idleModeMobilityControlInfo;

2>	if the t320 is included:

3>	start timer T320, with the timer value set according to the value of t320;

1>	else:

2>	apply the cell reselection priority information broadcast in the system information;

1>	if the RRCConnectionRelease message includes the measIdleConfig:

2>	clear VarMeasIdleConfig and VarMeasIdleReport;

2>	store the received measIdleDuration in VarMeasIdleConfig;

2>	start T331 with the value of measIdleDuration;

2>	if the measIdleConfig contains measIdleCarrierListEUTRA:

3>	store the received measIdleCarrierListEUTRA in VarMeasIdleConfig;

3>	start performing idle mode measurements as specified in 5.6.20;

NOTE 2:	If the measIdleConfig does not contain measIdleCarrierListEUTRA, UE may receive measIdleCarrierListEUTRA as specified in 5.2.2.12.

1>	for NB-IoT, if the RRCConnectionRelease message includes the redirectedCarrierInfo:

2>	if the redirectedCarrierOffsetDedicated is included in the redirectedCarrierInfo:

3>	store the dedicated offset for the frequency in redirectedCarrierInfo;

3>	start timer T322, with the timer value set according to the value of T322 in redirectedCarrierInfo;

1>	if the releaseCause received in the RRCConnectionRelease message indicates loadBalancingTAURequired:

2>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'load balancing TAU required';

1>	else if the releaseCause received in the RRCConnectionRelease message indicates cs-FallbackHighPriority:

2>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'CS Fallback High Priority';

1>	else:

2>	if the waitTime is present:

3>	start timer T302, with the timer value set according to the waitTime;

3>	inform the upper layer that access barring is applicable for all access categories except categories '0' and '2';

2>	if the extendedWaitTime is present; and

2>	if the UE supports delay tolerant access or the UE is a NB-IoT UE:

3>	forward the extendedWaitTime to upper layers;

2>	if the extendedWaitTime-CPdata is present and the NB-IoT UE only supports the Control Plane CIoT EPS optimisation:

3>	forward the extendedWaitTime-CPdata to upper layers;

2>	if the releaseCause received in the RRCConnectionRelease message indicates rrc-Suspend:

3>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'RRC suspension';

2>	else if rrc-InactiveConfig is included:

3>	perform the actions upon entering RRC_INACTIVE as specified in 5.3.8.7;

2>	else:

3>	perform the actions upon leaving RRC_CONNECTED or RRC_INACTIVE as specified in 5.3.12, with  release cause 'other';

8.1.5.5.1.3	Test Description

8.1.5.5.1.3.1	Pre-test conditions

System Simulator:

-	E-UTRA Cell 1 is the serving cell and NR Cell 1 is the suitable neighbour cell.

-	System information combination 31 as defined in TS 36.508 [7] clause 4.4.3.1 is used in the E-UTRA cell.

-	System information combination NR-6 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in the NR cell.

UE:

-	In E-UTRA RRC_CONNECTED state using generic procedure parameters Connectivity (E-UTRA/EPC) and Unrestricted nr PDN (On)in accordance with the procedure described in TS 38.508-1 [4], clause 4.5.4.

Preamble:

-	With NR Cell 1 "Serving cell" and E-UTRA Cell 1 "Non-suitable "Off" cell", the UE is brought to state 1N-A, RRC_IDLE Connectivity (NR), in accordance with the procedure described in TS 38.508-1 [4], Table 4.5.2.2-2. 5G-GUTI and ngKSI are assigned and security context established.

-	the UE is switched-off

-	With E-UTRA Cell 1  and NR Cell 1 power levels set according to T0, the UE is brought to state Generic RB Established (state 3) according to TS 36.508 [7] clause 4.5.3. 4G GUTI and eKSI are assigned and security context established

8.1.5.5.1.3.2	Test procedure sequence

Tables 8.1.5.5.1.3.2-0/ 8.1.5.5.1.3.2-0A illustrates the downlink power levels to be applied for NR Cell 1, E-UTRA Cell 1 for the test execution.

Table 8.1.5.5.1.3.2-0: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | E-UTRA Cell 1 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz |  | -85 |  |
|  | SS/PBCH SSS EPRE | dBm/SCS | ”Off” | - |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz |  | -85 |  |
|  | SS/PBCH SSS EPRE | dBm/SCS | -91 | - |  |

Table 8.1.5.5.1.3.2-0A: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | E-UTRA Cell 1 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz |  | -85 |  |
|  | SS/PBCH SSS EPRE | dBm/SCS | ”Off” | - |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz |  | -96 |  |
|  | SS/PBCH SSS EPRE | dBm/SCS | -91 | - |  |

Table 8.1.5.5.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | The SS changes Cell parameters according to the row “T1” in table 8.1.5.5.1.3.2-0/0A. | - | - | - | - |
| 1 | The SS transmits an RRCConnectionRelease message indicating redirection to NR Cell 1. | - | E-UTRA RRC: RRCConnectionRelease | - | - |
| 2-14a1 | Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.9 indicate that the UE has performed mobility registration updating when it camps on NR Cell 1? | - | - | 1 | P |

8.1.5.5.1.3.3	Specific message contents

Table 8.1.5.5.1.3.3-1: RRCConnectionRelease (step 1, Table 8.1.5.5.1.3.2-1)

| Derivation Path: TS 36.508 table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| redirectedCarrierInfo CHOICE { |  |  |  |
| nr-r15 SEQUENCE { |  |  |  |
| carrierFreq-r15 | ARFCN of NR Cell 1 |  |  |
| subcarrierSpacingSSB-r15 | kHz15 |  | FR1_FDD |
|  | kHz30 |  | FR1_TDD |
|  | kHz120 |  | FR2_TDD |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.1.5.5.2	Redirection to NR / From E-UTRA / MPS Priority Indication / Success

8.1.5.5.2.1	Test Purpose (TP)

(1)

with { UE in  E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives an RRCConnectionRelease message including MPS priority indication and a redirectedCarrierInfo for redirection to an NR cell }

then { UE enters E-UTRA RRC_IDLE state and performs redirection to NR cell }

}

8.1.5.5.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in TS 36.331 clause 5.3.3.3, TS 38.331 clause 5.3.8.3, and TS 38.304 clause 5.2.6. Unless otherwise stated these are Rel-16 requirements.

[TS 36.331, clause 5.3.3.3]

The UE shall set the contents of RRCConnectionRequest message as follows:

1>	if the UE is connected to EPC:

2>	set the ue-Identity as follows:

3>	if upper layers provide an S-TMSI:

4>	set the ue-Identity to the value received from upper layers;

3>	else:

4>	draw a random value in the range 0 .. 240-1 and set the ue-Identity to this value;

NOTE 1:	Upper layers provide the S-TMSI if the UE is registered in the TA of the current cell.

2>	if the establishment of the RRC connection is the result of release with redirect with mpsPriorityIndication (either in NR or E-UTRAN):

3>	set the establishmentCause to highPriorityAccess;

[TS 38.331, clause 5.3.8.3]

The UE shall:

1>	delay the following actions defined in this clause 60ms from the moment the RRCRelease message was received or optionally when lower layers indicate that the receipt of the RRCRelease message has been successfully acknowledged, whichever is earlier;

1>	stop timer T320, if running;

1>	if the RRCRelease message includes redirectedCarrierInfo indicating redirection to eutra:

2>	if cnType is included:

3>	after the cell selection, indicate the available CN Type(s) and the received cnType to upper layers;

NOTE:	Handling the case if the E-UTRA cell selected after the redirection does not support the core network type specified by the cnType, is up to UE implementation.

[TS 38.304, clause 5.2.6]

At reception of RRCRelease message to transition the UE to RRC_IDLE or RRC_INACTIVE, UE shall attempt to camp on a suitable cell according to redirectedCarrierInfo if included in the RRCRelease message used for this transition. If the UE cannot find a suitable cell, the UE is allowed to camp on any suitable cell of the indicated RAT. If the RRCRelease message does not contain the redirectedCarrierInfo, UE shall attempt to select a suitable cell on an NR carrier. If no suitable cell is found according to the above, the UE shall perform cell selection using stored information in order to find a suitable cell to camp on.

When returning to RRC_IDLE state after UE moved to RRC_CONNECTED state from camped on any cell state, UE shall attempt to camp on an acceptable cell according to redirectedCarrierInfo, if included in the RRCRelease message. If the UE cannot find an acceptable cell, the UE is allowed to camp on any acceptable cell of the indicated RAT. If the RRCRelease message does not contain redirectedCarrierInfo UE shall attempt to select an acceptable cell on an NR frequency. If no acceptable cell is found according to the above, the UE shall continue to search for an acceptable cell of any PLMN in state any cell selection.

8.1.5.5.2.3	Test Description

8.1.5.5.2.3.1	Pre-test conditions

System Simulator:

-	E-UTRA Cell 1 is the serving cell and NR Cell 1 is the suitable neighbour cell.

-	System information combination 31 as defined in TS 36.508 [7] clause 4.4.3.1 is used in the E-UTRA cell.

-	System information combination NR-6 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in the NR cell.

UE:

-	In E-UTRA RRC_CONNECTED state using generic procedure parameters Connectivity (E-UTRA/EPC) and Unrestricted nr PDN (On)in accordance with the procedure described in TS 38.508-1 [4], clause 4.5.4.

Preamble:

-	With NR Cell 1 "Serving cell" and E-UTRA Cell 1 "Non-suitable "Off" cell", the UE is brought to state 1N-A, RRC_IDLE Connectivity (NR), in accordance with the procedure described in TS 38.508-1 [4], Table 4.5.2.2-2. 5G-GUTI and ngKSI are assigned and security context established.

-	the UE is switched-off

-	With E-UTRA Cell 1  and NR Cell 1 power levels set according to T0, the UE is brought to state Generic RB Established (state 3) according to TS 36.508 [7] clause 4.5.3. 4G GUTI and eKSI are assigned and security context established

8.1.5.5.2.3.2	Test procedure sequence

Tables 8.1.5.5.2.3.2-1/ 8.1.5.5.2.3.2-2 illustrates the downlink power levels to be applied for NR Cell 1, E-UTRA Cell 1 for the test execution.

Table 8.1.5.5.2.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | E-UTRA Cell 1 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz |  | -85 |  |
|  | SS/PBCH SSS EPRE | dBm/SCS | ”Off” | - |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz |  | -85 |  |
|  | SS/PBCH SSS EPRE | dBm/SCS | -91 | - |  |

Table 8.1.5.5.2.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | E-UTRA Cell 1 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz |  | -85 |  |
|  | SS/PBCH SSS EPRE | dBm/SCS | ”Off” | - |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz |  | -96 |  |
|  | SS/PBCH SSS EPRE | dBm/SCS | -91 | - |  |

Table 8.1.5.5.2.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell parameters according to the row “T1” in table 8.1.5.5.2.3.2-1/2. | - | - | - | - |
| 2 | The SS transmits an RRCConnectionRelease message indicating mpsPriorityIndication-r16 and redirection to NR Cell 1. | <-- | E-UTRA RRC: RRCConnectionRelease | - | - |
| 3 | Check: Does the UE transmit an RRCSetupRequest Message with establishmentCause to highPriorityAccess. | --> | NR RRC: RRCSetupRequest | 1 | P |
| 4-8 | Steps 2 to 14b2b1 of the generic test procedure in TS 38.508-1 [4] subclause 4.9.9.2.2 are performed on NR Cell 1. | - | - | - | - |

8.1.5.5.2.3.3	Specific message contents

Table 8.1.5.5.2.3.3-1: RRCConnectionRelease (step 2, Table 8.1.5.5.2.3.2-3)

| Derivation Path: TS 36.508 table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| redirectedCarrierInfo CHOICE { |  |  |  |
| nr-r15 SEQUENCE { |  |  |  |
| carrierFreq-r15 | ARFCN of NR Cell 1 |  |  |
| subcarrierSpacingSSB-r15 | kHz15 |  | FR1_FDD |
|  | kHz30 |  | FR1_TDD |
|  | kHz120 |  | FR2_TDD |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| mpsPriorityIndication-r16 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FR1_FDD | FDD frequency range < 6GHz |
| FR1_TDD | TDD frequency range < 6GHz |
| FR2_TDD | TDD frequency range > 6GHz |

Table 8.1.5.5.2.3.3-2: RRCSetupRequest (step 3, Table 8.1.5.5.2.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupRequest ::= SEQUENCE { |  |  |  |
| rrcSetupRequest SEQUENCE { |  |  |  |
| establishmentCause | highPriorityAccess |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.1.5.6	Radio link failure

##### 8.1.5.6.1	Radio link failure / RRC connection re-establishment success

8.1.5.6.1.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE detecting physical layer recovery while T310 was running }

then { UE resumes the RRC connection without explicit signalling }

}

(2)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE detecting physical layer problems }

then { UE shall start timer T310 and UE does not initiate any RRC connection re-establishment procedure before expiring of timer T310 }

}

(3)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE detecting radio link failure on expiring of timer T310 }

then { UE starts timer T311 and UE initiates the RRC connection re-establishment procedure }

}

(4)

with { UE having initiated RRC connection re-establishment procedure and timer T311 running }

ensure that {

when { UE successfully completes the RRC connection re-establishment procedure }

then { UE is in NR RRC_CONNECTED state }

}

(5)

with { UE in NR RRC_CONNECTED state and upon detecting radio link failure UE sends RRCReestablishmentRequest message }

ensure that {

when { UE receives a RRCSetup message }

then { UE completes RRC setup procedure by sending RRCSetupComplete message }

}

8.1.5.6.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331 clauses 5.3.3.4, 5.3.7.2, 5.3.7.8, 5.3.10.1, 5.3.10.2 and 5.3.10.3.

[TS 38.331 clause 5.3.3.4]

The UE shall perform the following actions upon reception of the RRCSetup:

1>	if the RRCSetup is received in response to an RRCReestablishmentRequest; or

1>	if the RRCSetup is received in response to an RRCResumeRequest or RRCResumeRequest1:

2>	discard any stored UE Inactive AS context and suspendConfig;

2>	discard any current AS security context including the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key;

2>	release radio resources for all established RBs except SRB0, including release of the RLC entities, of the associated PDCP entities and of SDAP;

2>	release the RRC configuration except for the default L1 parameter values, default MAC Cell Group configuration and CCCH configuration;

2>	indicate to upper layers fallback of the RRC connection;

2>	stop timer T380, if running;

1>	perform the cell group configuration procedure in accordance with the received masterCellGroup and as specified in 5.3.5.5;

1>	perform the radio bearer configuration procedure in accordance with the received radioBearerConfig and as specified in 5.3.5.6;

1>	if stored, discard the cell reselection priority information provided by the cellReselectionPriorities or inherited from another RAT;

1>	stop timer T300, T301 or T319 if running;

1>	if T390 is running:

2>	stop timer T390 for all access categories;

2>	perform the actions as specified in 5.3.14.4;

1>	if T302 is running:

2>	stop timer T302;

2>	perform the actions as specified in 5.3.14.4;

1>	stop timer T320, if running;

1>	if the RRCSetup is received in response to an RRCResumeRequest, RRCResumeRequest1 or RRCSetupRequest:

2>	enter RRC_CONNECTED;

2>	stop the cell re-selection procedure;

1>	consider the current cell to be the PCell;

1>	set the content of RRCSetupComplete message as follows:

2>	if upper layers provide a 5G-S-TMSI:

3>	if the RRCSetup is received in response to an RRCSetupRequest:

4>	set the ng-5G-S-TMSI-Value to ng-5G-S-TMSI-Part2;

3>	else:

4>	set the ng-5G-S-TMSI-Value to ng-5G-S-TMSI;

2>	set the selectedPLMN-Identity to the PLMN selected by upper layers (TS 24.501 [23]) from the PLMN(s) included in the plmn-IdentityList in SIB1;

2>	if upper layers provide the 'Registered AMF':

3>	include and set the registeredAMF as follows:

4>	if the PLMN identity of the 'Registered AMF' is different from the PLMN selected by the upper layers:

5>	include the plmnIdentity in the registeredAMF and set it to the value of the PLMN identity in the 'Registered AMF' received from upper layers;

4>	set the amf-Identifier to the value received from upper layers;

3>	include and set the guami-Type to the value provided by the upper layers;

2>	if upper layers provide one or more S-NSSAI (see TS 23.003 [21]):

3>	include the s-NSSAI-List and set the content to the values provided by the upper layers;

2>	set the dedicatedNAS-Message to include the information received from upper layers;

1>	submit the RRCSetupComplete message to lower layers for transmission, upon which the procedure ends

[TS 38.331 clause 5.3.7.2]

The UE initiates the procedure when one of the following conditions is met:

1>	upon detecting radio link failure of the MCG, in accordance with 5.3.10; or

1>	upon re-configuration with sync failure of the MCG, in accordance with sub-clause 5.3.5.8.3; or

1>	upon mobility from NR failure, in accordance with sub-clause 5.4.3.5; or

1>	upon integrity check failure indication from lower layers concerning SRB1 or SRB2, except if the integrity check failure is detected on the RRCReestablishment message; or

1>	upon an RRC connection reconfiguration failure, in accordance with sub-clause 5.3.5.8.2.

Upon initiation of the procedure, the UE shall:

1>	stop timer T310, if running;

1>	stop timer T304, if running;

1>	start timer T311;

1>	suspend all RBs, except SRB0;

1>	reset MAC;

1>	release the MCG SCell(s), if configured;

1>	release spCellConfig;

1>	release delayBudgetReportingConfig, if configured, and stop timer T342, if running;

1>	release overheatingAssistanceConfig, if configured, and stop timer T345, if running;

1>	perform cell selection in accordance with the cell selection process as specified in TS 38.304 [20], clause 5.2.6.

[TS 38.331 clause 5.3.7.8]

The UE shall:

1>	perform the RRC connection establishment procedure as specified in 5.3.3.4.

[TS 38.331 clause 5.3.10.1]

The UE shall:

1>	upon receiving N310 consecutive "out-of-sync" indications for the SpCell from lower layers while neither T300, T301, T304, T311 nor T319 are running:

2>	start timer T310 for the corresponding SpCell.

[TS 38.331 clause 5.3.10.2]

Upon receiving N311 consecutive "in-sync" indications for the SpCell from lower layers while T310 is running, the UE shall:

1>	stop timer T310 for the corresponding SpCell.

NOTE 1:	In this case, the UE maintains the RRC connection without explicit signalling, i.e. the UE maintains the entire radio resource configuration.

NOTE 2:	Periods in time where neither "in-sync" nor "out-of-sync" is reported by L1 do not affect the evaluation of the number of consecutive "in-sync" or "out-of-sync" indications.

[TS 38.331 clause 5.3.10.3]

The UE shall:

1>	upon T310 expiry in PCell; or

1>	upon random access problem indication from MCG MAC while neither T300, T301, T304, T311 nor T319 are running; or

1>	upon indication from MCG RLC that the maximum number of retransmissions has been reached:

2>	if the indication is from MCG RLC and CA duplication is configured and activated, and for the corresponding logical channel allowedServingCells only includes SCell(s):

3>	initiate the failure information procedure as specified in 5.7.5 to report RLC failure.

2>	else:

3>	consider radio link failure to be detected for the MCG i.e. RLF;

3>	if AS security has not been activated:

4>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other';-

3>	else if AS security has been activated but SRB2 and at least one DRB have not been setup:

4>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure';

3>	else:

4>	initiate the connection re-establishment procedure as specified in 5.3.7.

8.1.5.6.1.3	Test description

8.1.5.6.1.3.1	Pre-test conditions

System Simulator:

-	3 NR Cells. NR Cell 1 is the serving cell, NR Cell 2 and NR Cell 4 are the intra-frequency neighbour cell.

None.

Preamble:

-	The UE is in state 3N-A as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1.

8.1.5.6.1.3.2	Test procedure sequence

Table 8.1.5.6.1.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 4 |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | “Off” | “Off” | “Off” |
| T0A | SS/PBCH SSS EPRE | dBm/SCS | -88 | “Off” | “Off” |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | “Off” | -88 | -94 |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | “Off” | “Off” | -88 |
| Power level “Off” is defined in TS 38.508-1 [4] Table 6.2.2.1-3. |  |  |  |  |  |

Table 8.1.5.6.1.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 4 |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | “Off” | “Off” | “Off” |
| T0A | SS/PBCH SSS EPRE | dBm/SCS | -82 | “Off” | “Off” |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | “Off” | -82 | -91 |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | “Off” | “Off” | -82 |
| Power level “Off” is defined in TS 38.508-1 [4] Table 6.2.2.2-2. |  |  |  |  |  |

Table 8.1.5.6.1.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0A | The SS changes NR Cell 1 parameters according to the row "T0" in table Table 8.1.5.6.1.3.2-1/2 to set NR Cell 1 to Non-suitable “Off”. | - | - | - | - |
| 0B | The SS wait for 1,5 s (T310 (2s) is transmitted in SIB1). | - | - | - | - |
| 0C | The SS changes NR Cell 1 parameters according to the row "T0A" in table 8.1.5.6.1.3.2-1/2 making NR Cell 1 serving cell. | - | - | - | - |
| 0D | Check: For the next 5 s does the UE transmit any signalling message? | - | - | 1 | F |
| 0E | Check: Does the test result of generic test procedure in TS 38.508-1 [4] subclause 4.9.10 indicate that the UE is in NR RRC_CONNECTED state on NR Cell 1? | - | - | 1 | P |
| 1 | The SS changes NR Cell 1, NR Cell 2 and NR Cell 4 parameters according to the row "T1" in table 8.1.5.6.1.3.2-1/2 in order that the radio link quality of NR Cell 1 is degraded, and NR Cell 2 is suitable for camping. | - | - | - | - |
| 2 | Check: Does the UE initiate an RRC connection re-establishment procedure on Cell 1 or Cell 2. This is checked during the time T=T310? | - | - | 2 | F |
| 3 | Check: Does the UE send RRCReestablishmentRequest message on NR Cell 2? | --> | NR RRC: RRCReestablishmentRequest | 3 | P |
| 4 | The SS transmits RRCReestablishment message. | <-- | NR RRC: RRCReestablishment | - | - |
| 5 | The UE transmits RRCReestablishmentComplete message. | --> | NR RRC: RRCReestablishmentComplete | - | - |
| 6 | The SS transmits an RRCReconfiguration message to resume existing radio bearer. | <-- | NR RRC: RRCReconfiguration | - | - |
| 7 | Check: Does the UE transmit an RRCReconfigurationComplete message? | --> | NR RRC: RRCReconfigurationtComplete | 4 | P |
| 8 | The SS changes NR Cell 2 and NR Cell 4 parameters according to the row "T2" in table 8.1.5.6.1.3.2-1/2 in order that the radio link quality of NR Cell 2 is degraded and cell 4 is suitable for camping. | - | - | - | - |
| 9 | Check: Does the UE send RRCReestablishmentRequest message on NR Cell 4? | --> | NR RRC: RRCReestablishmentRequest | - | - |
| 10 | The SS transmits RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 11 | Check: Does the UE transmit a RRCSetupComplete message and a REGISTRATION REQUEST message indicating "mobility registration updating"? | --> | NR RRC: RRCSetupComplete 5GMM: REGISTRATION REQUEST | 5 | P |
| 11A | The SS transmits a DLInformationTransfer message and an AUTHENTICATION REQUEST message. | <-- | NR RRC: DLInformationTransfer NAS: AUTHENTICATION REQUEST | - | - |
| 11B | The UE transmits an ULInformationTransfer message and an AUTHENTICATION RESPONSE message. | --> | NR RRC: ULInformationTransfer 5GMM: AUTHENTICATION RESPONSE | - | - |
| 11C | The SS transmits a NAS SECURITY MODE COMMAND message to activate NAS security. | <-- | NR RRC: DLInformationTransfer 5GMM:SECURITY MODE COMMAND | - | - |
| 11D | The UE transmits a NAS SECURITY MODE COMPLETE message and establishes the initial security configuration. | --> | NR RRC: ULInformationTransfer 5GMM:SECURITY MODE COMPLETE | - | - |
| 12 | The SS transmits a SecurityModeCommand message. | <-- | NR RRC: SecurityModeCommand | - | - |
| 13 | The UE transmits a SecurityModeComplete message. | --> | NR RRC: SecurityModeComplete | - | - |
| 14 | The SS transmits an RRCReconfiguration message to establish SRB2 and DRB and a REGISTRATION ACCEPT message. | <-- | NR RRC: RRCReconfiguration 5GMM: REGISTRATION ACCEPT | - | - |
| - | EXCEPTION: Depending upon UE implementation, steps 15 and 16 can occur in any order | - | - | - | - |
| 15 | The UE transmits an RRCReconfigurationtComplete message. | --> | NR RRC: RRCReconfigurationtComplete | - | - |
| 16 | The UE sends a REGISTRATION COMPLETE. | --> | NR RRC: ULInformationTransfer 5GMM: REGISTRATION COMPLETE | - | - |

8.1.5.6.1.3.3	Specific message contents

Table 8.1.5.6.1.3.3-0: SIB1 for Cell 1 (all steps)

| Derivation Path: TS 38.508-1 [4] table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| ue-TimersAndConstants SEQUENCE { |  |  |  |
| t310 | ms2000 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.6.1.3.3-0A: RLF-TimersAndConstants(preamble)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-150 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLF-TimersAndConstants ::= SEQUENCE { |  |  |  |
| t310 | ms2000 |  |  |
| } |  |  |  |

Table 8.1.5.6.1.3.3-0B: RRCSetup (preamble)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-21 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetup ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcSetup SEQUENCE { |  |  |  |
| masterCellGroup SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| rlf-TimersAndConstants CHOICE { |  |  |  |
| setup | RLF-TimersAndConstants | Table 8.1.5.6.1.3.3-0A |  |
| } |  |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |
| initialDownlinkBWP SEQUENCE { |  |  |  |
| radioLinkMonitoringConfig CHOICE { |  |  |  |
| setup | RadioLinkMonitoringConfig specified in TS 38.508-1[4] Table 4.6.3-133 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.6.1.3.3-1: RRCReestablishmentRequest (step 3, Table 8.1.5.6.1.3.2-3)

| Derivation Path: TS 38.508-1 [4] table 4.6.1-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReestablishmentRequest ::= SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of NR Cell 1 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS |  |  |
| } |  |  |  |
| reestablishmentCause | otherFailure |  |  |
| } |  |  |  |

Table 8.1.5.6.1.3.3-1A: RRCReconfiguration (step 6, Table 8.1.5.6.1.3.2-3)

| Derivation Path: TS 38.508-1[4], table 4.6.1-13 with condition REEST |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |
| initialDownlinkBWP SEQUENCE { |  |  |  |
| radioLinkMonitoringConfig CHOICE { |  |  |  |
| setup | RadioLinkMonitoringConfig specified in TS 38.508-1[4] Table 4.6.3-133 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.6.1.3.3-2: RRCReestablishmentRequest (step 9, Table 8.1.5.6.1.3.2-3)

| Derivation Path: TS 38.508-1 [4] table 4.6.1-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReestablishmentRequest ::= SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of NR Cell 2 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS |  |  |
| } |  |  |  |
| reestablishmentCause | otherFailure |  |  |
| } |  |  |  |

Table 8.1.5.6.1.3.3-3: RRCReconfiguration (step 14, Table 8.1.5.6.1.3.2-3)

Derivation Path: TS 38.508-1[4], table 4.6.1-13 with condition NR

##### 8.1.5.6.2	Void

##### 8.1.5.6.3	Radio link failure / T311 expiry

8.1.5.6.3.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state and radio link failure was detected and UE attempts to select a suitable NR cell to re-establish the RRC connection }

ensure that {

when { UE cannot find a suitable NR cell within T311 }

then { UE does not try to re-establish the RRC connection and goes to NR RRC_IDLE state after T311 expired }

}

8.1.5.6.3.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331 clauses 5.3.7.2, 5.3.7.6, 5.3.7.8, 5.3.10.1, 5.3.10.3 and 5.3.5.5.7.

[TS 38.331 clause 5.3.7.2]

The UE initiates the procedure when one of the following conditions is met:

1>	upon detecting radio link failure of the MCG, in accordance with 5.3.10; or

1>	upon re-configuration with sync failure of the MCG, in accordance with sub-clause 5.3.5.8.3; or

1>	upon mobility from NR failure, in accordance with sub-clause 5.4.3.5; or

1>	upon integrity check failure indication from lower layers concerning SRB1 or SRB2, except if the integrity check failure is detected on the RRCReestablishment message; or

1>	upon an RRC connection reconfiguration failure, in accordance with sub-clause 5.3.5.8.2.

Upon initiation of the procedure, the UE shall:

1>	stop timer T310, if running;

1>	stop timer T304, if running;

1>	start timer T311;

1>	suspend all RBs, except SRB0;

1>	reset MAC;

1>	release the MCG SCell(s), if configured;

1>	release spCellConfig;

1>	release delayBudgetReportingConfig, if configured, and stop timer T342, if running;

1>	release overheatingAssistanceConfig, if configured, and stop timer T345, if running;

1>	perform cell selection in accordance with the cell selection process as specified in TS 38.304 [20], clause 5.2.6.

[TS 38.331 clause 5.3.7.7]

Upon T311 expiry, the UE shall:

1>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure'.

[TS 38.331 clause 5.3.7.8]

The UE shall:

1>	perform the RRC connection establishment procedure as specified in 5.3.3.4.

[TS 38.331 clause 5.3.10.1]

The UE shall:

1>	upon receiving N310 consecutive "out-of-sync" indications for the SpCell from lower layers while neither T300, T301, T304, T311 nor T319 are running:

2>	start timer T310 for the corresponding SpCell.

[TS 38.331 clause 5.3.10.3]

The UE shall:

1>	upon T310 expiry in PCell; or

1>	upon random access problem indication from MCG MAC while neither T300, T301, T304, T311 nor T319 are running; or

1>	upon indication from MCG RLC that the maximum number of retransmissions has been reached:

2>	if the indication is from MCG RLC and CA duplication is configured and activated, and for the corresponding logical channel allowedServingCells only includes SCell(s):

3>	initiate the failure information procedure as specified in 5.7.5 to report RLC failure.

2>	else:

3>	consider radio link failure to be detected for the MCG i.e. RLF;

3>	if AS security has not been activated:

4>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other';-

3>	else if AS security has been activated but SRB2 and at least one DRB have not been setup:

4>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure';

3>	else:

4>	initiate the connection re-establishment procedure as specified in 5.3.7.

[TS 38.331, 5.3.5.5.7]

The UE shall:

1>	if the SpCellConfig contains the rlf-TimersAndConstants:

2>	configure the RLF timers and constants for this cell group as specified in 5.3.5.5.6;

1>	else if rlf-TimersAndConstants is not configured for this cell group:

2>	use values for timers T301, T310, T311 and constants N310, N311, as included in ue-TimersAndConstants received in SIB1;

8.1.5.6.3.3	Test description

8.1.5.6.3.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

None.

Preamble:

-	The UE is in state 3N-A as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1.

8.1.5.6.3.3.2	Test procedure sequence

Table 8.1.5.6.3.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 |
| --- | --- | --- | --- |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | “Off” |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -88 |
| Power level “Off” is defined in TS 38.508-1 [4] Table 6.2.2.1-3. |  |  |  |

Table 8.1.5.6.3.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 |
| --- | --- | --- | --- |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | “Off” |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -82 |
| Power level “Off” is defined in TS 38.508-1 [4] Table 6.2.2.2-2. |  |  |  |

Table 8.1.5.6.3.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes NR Cell 1 parameters according to the row "T1" in table 8.1.5.6.3.3.2-1 in order that the radio link quality of NR Cell 1 is degraded. | - | - | - | - |
| 2 | Wait for 12s (T311 (10s) is transmitted in RRCSetup). | - | - | - | - |
| 3 | The SS changes NR Cell 1 parameters according to the row "T2" in table 8.1.5.6.3.3.2-1 making NR Cell 1 is suitable for camping. | - | - | - | - |
| 4 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] subclause 4.9.5 indicate that the UE is camped on NR Cell 1? | - | - | 1 | P |

8.1.5.6.3.3.3	Specific message contents

Table 8.1.5.6.3.3.3-1: RRCSetup(Preamble)

| Derivation Path: TS 38.508-1 [4] table 4.6.1-21 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetup ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcSetup SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig with condition SRB1 | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.6.3.3.3-2: CellGroupConfig (Table 8.1.5.6.3.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlf-TimersAndConstants SEQUENCE { |  |  |  |
| t311-v1530 | ms10000 |  |  |
| } |  |  |  |
| spCellConfigDedicated | ServingCellConfig |  |  |
| } |  |  |  |

Table 8.1.5.6.3.3.3-3: ServingCellConfig (Table 8.1.5.6.3.3.3-2: CellGroupConfig)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkDedicated |  |  |
| } |  |  |  |

Table 8.1.5.6.3.3.3-4: BWP-DownlinkDedicated (Table 8.1.5.6.3.3.3-3: ServingCellConfig)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkDedicated ::= SEQUENCE { |  |  |  |
| radioLinkMonitoringConfig | RadioLinkMonitoringConfig |  |  |
| } |  |  |  |

##### 8.1.5.6.4	Void

##### 8.1.5.6.5	NR CA / No Radio Link Failure on SCell / RRC Connection Continues on PCell

###### 8.1.5.6.5.1	NR CA / No Radio Link Failure on SCell / RRC Connection Continues on PCell / Intra-band Contiguous CA

8.1.5.6.5.1.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED, security activated and SCell configured }

ensure that {

when { the SCell becomes out of coverage for the UE }

then { UE does not consider radio link failure to be detected and stay in NR RRC_CONNECTED state }

}

8.1.5.6.5.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331, clauses 5.3.5.5.6, 5.3.10.1 and 5.3.10.3. Unless otherwise stated these are Rel-15 requirements.

[TS 38.331, clause 5.3.5.5.6]

The UE shall:

1>	if the received rlf-TimersAndConstants is set to release:

2>	use values for timers T301, T310, T311 and constants N310, N311, as included in ue-TimersAndConstants received in SIB1;

1>	else:

2>	(re-)configure the value of timers and constants in accordance with received rlf-TimersAndConstants;

2>	stop timer T310 for this cell group, if running;

2>	reset the counters N310 and N311.

[TS 38.331, clause 5.3.10.1]

The UE shall:

1>	upon receiving N310 consecutive "out-of-sync" indications for the SpCell from lower layers while neither T300, T301, T304, T311 nor T319 are running:

2>	start timer T310 for the corresponding SpCell.

[TS 38.331, clause 5.3.10.3]

The UE shall:

1>	upon T310 expiry in PCell; or

1>	upon random access problem indication from MCG MAC while neither T300, T301, T304, T311 nor T319 are running; or

1>	upon indication from MCG RLC that the maximum number of retransmissions has been reached:

2>	if the indication is from MCG RLC and CA duplication is configured and activated, and for the corresponding logical channel allowedServingCells only includes SCell(s):

3>	initiate the failure information procedure as specified in 5.7.5 to report RLC failure.

2>	else:

3>	consider radio link failure to be detected for the MCG, i.e. MCG RLF;

3>	if AS security has not been activated:

4>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other';-

3>	else if AS security has been activated but SRB2 and at least one DRB have not been setup:

4>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure';

3>	else:

4>	initiate the connection re-establishment procedure as specified in 5.3.7.

The UE shall:

1>	upon T310 expiry in PSCell; or

1>	upon random access problem indication from SCG MAC; or

1>	upon indication from SCG RLC that the maximum number of retransmissions has been reached:

2>	if the indication is from SCG RLC and CA duplication is configured and activated, and for the corresponding logical channel allowedServingCells only includes SCell(s):

3>	initiate the failure information procedure as specified in 5.7.5 to report RLC failure.

2>	else:

3>	consider radio link failure to be detected for the SCG, i.e. SCG RLF;

3>	initiate the SCG failure information procedure as specified in 5.7.3 to report SCG radio link failure.

8.1.5.6.5.1.3	Test description

8.1.5.6.5.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1, and NR Cell 3

-	NR Cell 1 is PCell

-	NR Cell 3 is SCell

-	System information combination NR-4 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

None.

Preamble:

-	The UE is in 5GS state 3N-A with UE test loop mode B active if pc_IP_Ping is set to FALSE according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3 on NR Cell 1.

8.1.5.6.5.1.3.2	Test procedure sequence

Table 8.5.1.7.1.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.5.6.5.1.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -88 | NR Cell1 and NR Cell 3 are available |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -88 | “Off” | Only NR Cell 1 is available. |

Table 8.1.5.6.5.1.3.2-1A: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -95 | -95 | NR Cell1 and NR Cell 3 are available |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -95 | “Off” | Only NR Cell 1 is available. |

Table 8.1.5.6.5.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message on NR Cell 1 to configure NR Cell 3 as SCell. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message on NR Cell 1. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 2A | The SS transmits an Activation MAC control element to activate SCell. | <-- | MAC PDU (SCell Activation/Deactivation MAC CE of one octet (C1=1)) | - | - |
| 3 | Wait for 1s. | - | - | - | - |
| 4 | The SS changes power levels according to row "T1" in Table 8.1.5.6.5.1.3.2-1/1A. | - | - | - | - |
| 5 | Wait for 1s. | - | - | - | - |
| 6 | Check: Does the UE transmit an RRCReestablishmentRequest message on NR Cell 1 within next 2 seconds? | --> | NR RRC: RRCReestablishmentRequest | 1 | F |
| 7 | The SS changes power levels according to row "T0" in Table 8.1.5.6.5.1.3.2-1/1A. | - | - | - | - |
| 8 | Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on NR Cell 3? | - | - | 1 | - |
| 9 | The SS transmits an RRCReconfiguration message containing an sCellToReleaseList with SCell NR Cell 3. | <-- | NR RRC: RRCReconfiguration | - | - |
| 10 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationtComplete | - | - |

8.1.5.6.5.1.3.3	Specific message contents

Table 8.1.5.6.5.1.3.3-1: RRCReconfiguration (step 1, Table 8.1.5.6.5.1.3.2-2)

Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 with condition SCell_add

Table 8.1.5.6.5.1.3.3-2: Void

Table 8.1.5.6.5.1.3.3-3: Void

Table 8.1.5.6.5.1.3.3-4: RRCReconfiguration (Step 9, Table 8.1.5.6.5.1.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with Condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup SEQUENCE { |  |  |  |
| sCellToAddModList | Not present |  |  |
| sCellToReleaseList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellIndex { | 1 entry |  |  |
| SCellIndex[1] | SCellIndex of Cell 3 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

###### 8.1.5.6.5.2	NR CA / No Radio Link Failure on SCell / RRC Connection Continues on PCell / Inter-band CA

8.1.5.6.5.2.1	Test Purpose (TP)

Same as TC 8.1.5.6.5.1 but applied to Inter-band CA Test Case.

8.1.5.6.5.2.2	Conformance Requirements

Same as TC 8.1.5.6.5.1 but applied to Inter-band CA Test Case.

8.1.5.6.5.2.3	Test Description

8.1.5.6.5.2.3.1	Pre-test conditions

Same as TC 8.1.5.6.5.1 but with the following differences:

-	CA configuration: Inter-band CA replaces Intra-band Contiguous CA.

-	Cells configuration: Cell 10 replaces Cell 3.

-	Cell 10 is an Inactive SCell according to TS 38.508-1 [4] clause 6.3.1.

8.1.5.6.5.2.3.2	Test procedure sequence

Same as TC 8.1.5.6.5.1 with the following differences:

-	CA configuration: Inter-band CA replaces Intra-band Contiguous CA.

-	Cells configuration: Cell 10 replaces Cell 3.

8.1.5.6.5.2.3.3	Specific message contents

Same as TC 8.1.5.6.5.1 but applied to Inter-band CA Test Case.

###### 8.1.5.6.5.3	NR CA / No Radio Link Failure on SCell / RRC Connection Continues on PCell / Intra-band non-Contiguous CA

8.1.5.6.5.3.1	Test Purpose (TP)

Same as TC 8.1.5.6.5.1 but applied to Intra-band non-Contiguous CA Test Case.

8.1.5.6.5.3.2	Conformance Requirements

Same as TC 8.1.5.6.5.1 but applied to Intra-band non-Contiguous CA Test Case.

8.1.5.6.5.3.3	Test Description

8.1.5.6.5.3.3.1	Pre-test conditions

Same as TC 8.1.5.6.5.1 but with the following differences:

-	CA configuration: Intra-band non-Contiguous CA replaces Intra-band Contiguous CA.

8.1.5.6.5.3.3.2	Test procedure sequence

Same as TC 8.1.5.6.5.1 with the following differences:

-	CA configuration: Intra-band non-Contiguous CA replaces Intra-band Contiguous CA.

8.1.5.6.5.3.3.3	Specific message contents

Same as TC 8.1.5.6.5.1 but applied to Intra-band non-Contiguous CA Test Case.

##### 8.1.5.6.6	Radio link failure / Shared spectrum / LBT Failure

8.1.5.6.6.1	Radio link failure / LBT Failure

8.1.5.6.6.1.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { a UE detects consistent uplink LBT failures }

then { the UE detects radio link failure and performs connection re-establishment procedure }

}

8.1.5.6.6.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.321, clauses 5.21.1, 5.21.2, and TS 38.331, clauses 5.3.7.1, 5.3.7.2, 5.3.10.3 and 5.3.10.4. Unless otherwise stated these are Rel-16 requirements.

[TS 38.321, clause 5.21.1]

The lower layer may perform an LBT procedure, see TS 37.213 [18], according to which a transmission is not performed by lower layers if the channel is identified as being occupied. When lower layer performs an LBT procedure before a transmission and the transmission is not performed, an LBT failure indication is sent to the MAC entity from lower layers.

[TS 38.321, clause 5.21.2]

The MAC entity may be configured by RRC with a consistent LBT failure recovery procedure. Consistent LBT failure is detected per UL BWP by counting LBT failure indications, for all UL transmissions, from the lower layers to the MAC entity.

RRC configures the following parameters in the lbt-FailureRecoveryConfig:

-	lbt-FailureInstanceMaxCount for the consistent LBT failure detection;

-	lbt-FailureDetectionTimer for the consistent LBT failure detection;

The following UE variable is used for the consistent LBT failure detection procedure:

-	LBT_COUNTER (per Serving Cell): counter for LBT failure indication which is initially set to 0.

For each activated Serving Cell configured with lbt-FailureRecoveryConfig, the MAC entity shall:

1>	if LBT failure indication has been received from lower layers:

2>	start or restart the lbt-FailureDetectionTimer;

2>	increment LBT_COUNTER by 1;

2>	if LBT_COUNTER >= lbt-FailureInstanceMaxCount:

3>	trigger consistent LBT failure for the active UL BWP in this Serving Cell;

3>	if this Serving Cell is the SpCell:

4>	if consistent LBT failure has been triggered in all UL BWPs configured with PRACH occasions on same carrier in this Serving Cell:

5>	indicate consistent LBT failure to upper layers.

[TS 38.331, clause 5.3.7.1]

The purpose of this procedure is to re-establish the RRC connection. A UE in RRC_CONNECTED, for which AS security has been activated with SRB2 and at least one DRB setup or, for IAB, SRB2, may initiate the procedure in order to continue the RRC connection. The connection re-establishment succeeds if the network is able to find and verify a valid UE context or, if the UE context cannot be retrieved, and the network responds with an RRCSetup according to clause 5.3.3.4.

[TS 38.331, clause 5.3.7.2]

The UE initiates the procedure when one of the following conditions is met:

1>	upon detecting radio link failure of the MCG and t316 is not configured, in accordance with 5.3.10; or

[TS 38.331, clause 5.3.10.3]

The UE shall:

…

1>	else:

…

2>	upon consistent uplink LBT failure indication from MCG MAC while T304 is not running:

…

3>	else:

4>	consider radio link failure to be detected for the MCG, i.e. MCG RLF;

4>	discard any segments of segmented RRC messages stored according to 5.7.6.3;

…

4>	else:

5>	store the radio link failure information in the VarRLF-Report as described in clause 5.3.10.5;

…

5>	else:

6>	initiate the connection re-establishment procedure as specified in clause 5.3.7.

[TS 38.331, clause 5.3.10.4]

The UE shall set the rlf-Cause in the VarRLF-Report as follows:

…

1>	else if the UE declares radio link failure due to consistent uplink LBT failures:

2>	set the rlf-Cause as lbtFailure;

8.1.5.6.6.1.3	Test description

8.1.5.6.6.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

-	NR Cell 1 is configured to operate in shared spectrum.

UE:

-	None.

Preamble:

-	The UE is in 5GS state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3 and Test Loop Function (On) with UE test loop mode B is established.

8.1.5.6.6.1.3.2	Test procedure sequence

Table 8.1.5.6.6.1.3.2-0 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble.

Table 8.1.5.6.6.1.3.2-0: Cell configuration power level changes over time for conducted test environment

|  | Parameter | Unit | NR Cell 1 |
| --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -75 |
|  | Noc | dBm/SCS | -82 |

Table 8.1.5.6.6.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits one IP packet to the UE on the DRB associated with the first PDU session on NR Cell 1. | - | - | - | - |
| 1A | The SS adjusts the SS/PBCH EPRE and Noc levels according to row "T1" in table 8.1.5.6.6.1.3.2-0. | - | - | - | - |
| 2 | UE detects consistent LBT failures while attempting to loop back the data received at step 1. | - | - | - | - |
| 2A | 80ms after step 1A, SS adjusts the SS/PBCH EPRE and levels according to row "T0" in table 8.1.5.6.6.1.3.2-0. (Note 1) | - | - | - | - |
| 3 | Check: Does the UE send RRCReestablishmentRequest message on NR Cell 1? | --> | NR RRC: RRCReestablishmentRequest | 1 | P |
| 4 | The SS transmits RRCReestablishment message. | <-- | NR RRC: RRCReestablishment | - | - |
| 5 | The UE transmits an RRCReestablishmentComplete message. | --> | NR RRC: RRCReestablishmentComplete | - | - |
| 6 | The SS transmits an RRCReconfiguration message to establish SRB2 and DRB. | <-- | NR RRC: RRCReconfiguration | - | - |
| 7 | The UE transmits an RRCReconfigurationtComplete message. | --> | NR RRC: RRCReconfigurationtComplete | - | - |
| Note 1: 80ms is sufficient for UE to detect consistent uplink LBT failures as IE lbt-FailureDetectionTimer-r16 is set to ms80 in preamble. |  |  |  |  |  |

8.1.5.6.6.1.3.3	Specific message contents

Table 8.1.5.6.6.1.3.3-1: RRCSetup (preamble, Table 8.1.5.6.6.1.3.2-1)

| Derivation Path: 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetup ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcSetup SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.6.6.1.3.3-2: CellGroupConfig (Table 8.1.5.6.6.1.3.3-1: RRCSetup)

| Derivation Path: 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| cellGroupConfig::= SEQUENCE { |  |  |  |
| cellGroupId | 0 |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |
| servingCellConfig | ServingCellConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.6.6.1.3.3-3: ServingCellConfig (Table 8.1.5.6.6.1.3.3-2: CellGroupConfig)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkDedicated with condition SharedSpectrum |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.6.6.1.3.3-4: Void

Table 8.1.5.6.6.1.3.3-5: Void

Table 8.1.5.6.6.1.3.3-6: RRCReestablishmentRequest (step 3, Table 8.1.5.6.6.1.3.2-1)

| Derivation Path: TS 38.508-1 [4] table 4.6.1-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReestablishmentRequest ::= SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of NR Cell 1 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS |  |  |
| } |  |  |  |
| reestablishmentCause | otherFailure |  |  |
| } |  |  |  |

Table 8.1.5.6.6.1.3.3-7: RRCReconfiguration (step 6, Table 8.1.5.6.6.1.3.2-1)

Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 with condition REEST

Table 8.1.5.6.6.1.3.3-8: SIB1 for NR Cell 1 (All steps, Table 8.1.5.6.6.1.3.2-1)

| Derivation path: TS 38.508-1 [4] table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| servingCellConfigCommon |  | ServingCellConfigCommonSIB with condition SharedSpectrum |  |  |
| } |  |  |  |  |

#### 8.1.5.7	Failure information

##### 8.1.5.7.1	Failure information / RLC failure / MCG

###### 8.1.5.7.1.1	Failure information / RLC failure / MCG / Intra-band Contiguous CA

8.1.5.7.1.1.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { an indication from MCG RLC that the maximum number of retransmissions has been reached and when CA duplication is configured and activated and for the corresponding logical channel allowedServingCells only includes SCell(s) }

then { UE initiates the failure information procedure to report RLC failure via SRB1 }

}

8.1.5.7.1.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clauses 5.7.5.2 and 5.7.5.3. Unless otherwise stated these are Rel-15 requirements.

[TS 38.331, clause 5.3.10.3]

The UE shall:

…

1>	upon indication from MCG RLC that the maximum number of retransmissions has been reached:

2>	if the indication is from MCG RLC and CA duplication is configured and activated, and for the corresponding logical channel allowedServingCells only includes SCell(s):

3>	initiate the failure information procedure as specified in 5.7.5 to report RLC failure.

…

[TS 38.331, clause 5.7.5.2]

A UE initiates the procedure when there is a need inform the network about a failure detected by the UE. In particular, the UE initiates the procedure when the following condition is met:

1>	upon detecting failure for an RLC bearer, in accordance with 5.3.10.3;

Upon initiating the procedure, the UE shall:

1>	initiate transmission of the FailureInformation message as specified in 5.7.5.3;

[TS 38.331, clause 5.7.5.3]

The UE shall:

1>	if initiated to provide RLC failure information, set FailureInfoRLC-Bearer as follows:

2>	set logicalChannelIdentity to the logical channel identity of the failing RLC bearer;

2>	set cellGroupId to the cell group identity of the failing RLC bearer;

2>	set the failureType as rlc-failure;

1>	if used to inform the network about a failure for an MCG RLC bearer:

2>	submit the FailureInformation message to lower layers for transmission via SRB1;

…

8.1.5.7.1.1.3	Test description

8.1.5.7.1.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the PCell, NR Cell 3 is the SCell.

UE:

-	None.

Preamble:

Same Pre-Test conditions as in clause 7.1.3.0 except the following:

-	2 NR cells (NR Cell 1 as Pcell and NR Cell 3 as Scell)

-	NR Cell 1 is configured with DRBs in RLC AM mode

8.1.5.7.1.1.3.2	Test procedure sequence

Table 8.1.5.7.1.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message to add SCell (NR Cell 3) for the UE. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 2A | The SS transmits a SCell Activation/Deactivation MAC-CE to activate SCell (NR Cell 3). | <-- | MAC PDU (SCell Activation/Deactivation MAC CE of one octet (C1=1)) | - | - |
| 3 | The SS transmits an RRCReconfiguration message to reconfigure DRB #j for the UE | <-- | NR RRC: RRCReconfiguration | - | - |
| 4 | The UE transmits an RRCReconfigurationComplete message | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 5 | Void | - | - | - | - |
| 6 | Void | - | - | - | - |
| 7 | The SS stops sending MCG RLC acknowledgmentson PCell (NR Cell 1) and SCell (NR Cell 3) | - | - | - | - |
| 8 | SS transmits a periodic UL Grant on SCell (as PUCCH is not configured in the SCell), allowing the UE to loop back the PDCP PDU on SCell. | - |  | - | - |
| 8A | The SS transmits one PDCP PDU to the UE on DRB #j. | <- | PDCP PDU | - | - |
| - | EXCEPTION: Steps 9-9A occur in any sequence. | - | - | - | - |
| 9 | The UE loops back the PDCP PDU on PCell (NR Cell 1) on DRB #j | --> | PDCP PDU | - | - |
| 9A | The UE loops back the PDCP PDU on SCell (NR Cell 3) on DRB #j | --> | PDCP PDU | - | - |
| 10 | Check: Does the UE transmit a FailureInformation message with failureType set to ‘rlc-failure’? | --> | NR RRC: FailureInformation | 1 | P |
| 10A | SS resumes sending MCG RLC acknowledgments | - | - | - | - |
| 11 | The SS transmits an RRCReconfiguration message containing an sCellToReleaseList with SCell NR Cell 3. | <-- | NR RRC: RRCReconfiguration | - | - |
| 12 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |

8.1.5.7.1.1.3.3	Specific message contents

Table 8.1.5.7.1.1.3.3-1: RRCReconfiguration (Step 1, Table 8.1.5.7.1.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup SEQUENCE { |  |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig { | 1 entry |  |  |
| SCellConfig[1] SEQUENCE { |  | entry 1 |  |
| sCellIndex | SCellIndex for NR Cell 3 |  |  |
| sCellConfigCommon | ServingCellConfigCommon |  |  |
| sCellConfigDedicated | ServingCellConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.7.1.1.3.3-1A: ServingCellConfig (Table 8.1.5.7.1.1.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-167. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkDedicated |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.7.1.1.3.3-1B: BWP-UplinkDedicated (Table 8.1.5.7.1.1.3.3-1A)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicated ::= SEQUENCE { |  |  |  |
| pucch-Config | Not present |  |  |
| } |  |  |  |

Table 8.1.5.7.1.1.3.3-1C: ServingCellConfigCommon (Table 8.1.5.7.1.1.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-168. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| physCellId | Physical Cell Identity of NR Cell 3 |  |  |
| uplinkConfigCommon | UplinkConfigCommon |  |  |
| } |  |  |  |

Table 8.1.5.7.1.1.3.3-1D: UplinkConfigCommon (Table 8.1.5.7.1.1.3.3-1C)

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkConfigCommon ::= SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkCommon |  |  |
| } |  |  |  |

Table 8.1.5.7.1.1.3.3-1E: BWP-UplinkCommon (Table 8.1.5.7.1.1.3.3-1D)

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkCommon ::= SEQUENCE { |  |  |  |
| pucch-ConfigCommon | Not present |  |  |
| } |  |  |  |

Table 8.1.5.7.1.1.3.3-2: RRCReconfiguration (Step 3, Table 8.1.5.7.1.1.3.2-1)

| Derivation Path: TS 38.508-1[4], Table 4.6.1-13 with condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig-DRB | Table 8.1.5.7.1.1.3.3-3 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig-DRB | Table 8.1.5.7.1.1.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.7.1.1.3.3-3: RadioBearerConfig-DRB (Table 8.1.5.7.1.1.3.3-2)

| Derivation Path: TS 508-1 [4], Table 4.6.3-132 with condition DRB2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| drb-Identity | DRB-Identity with condition DRB j | j is the ID of the DRB established during the preamble which is allocated according to internal TTCN mapping |  |
| pdcp-Config | PDCP-Config |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.7.1.1.3.3-4: PDCP-Config (Table 8.1.5.7.1.1.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-99 with condition Split |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-Config ::= SEQUENCE { |  |  |  |
| moreThanOneRLC SEQUENCE { |  |  |  |
| primaryPath SEQUENCE { |  |  |  |
| logicalChannel | LogicalChannelIdentity with Condition DRB j |  |  |
| } |  |  |  |
| pdcp-Duplication | True |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.7.1.1.3.3-5: CellGroupConfig-DRB

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with Condition DRB j |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-BearerConfig { | 2 entries |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig-1 | entry 1 Table 8.1.5.7.1.1.3.3-6 Primary RLC |  |
| RLC-BearerConfig[2] | RLC-BearerConfig-2 | entry 1 Table 8.1.5.7.1.1.3.3-7 Secondary RLC |  |
| } |  |  |  |
| spCellConfig | Not present |  |  |
| } |  |  |  |

Table 8.1.5.7.1.1.3.3-6: RLC-BearerConfig-1 (Table 8.1.5.7.1.1.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-148 with Condition AM |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-BearerConfig ::= SEQUENCE { |  |  |  |
| logicalChannelIdentity | LogicalChannelIdentity with condition DRB j | ID of primary logical channel |  |
| servedRadioBearer CHOICE { |  |  |  |
| drb-Identity | DRB-Identity with condition DRB j |  |  |
| } |  |  |  |
| rlc-Config CHOICE { |  |  |  |
| am SEQUENCE { |  |  |  |
| ul-AM-RLC SEQUENCE { |  |  |  |
| maxRetxThreshold | t32 | To ensure RLC failure happens before RLF |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| mac-LogicalChannelConfig SEQUENCE { |  |  |  |
| ul-SpecificParameters SEQUENCE { |  |  |  |
| allowedServingCells SEQUENCE (SIZE (1..maxNrofServingCells-1)) OF ServCellIndex { | 1 entry |  |  |
| ServCellIndex[1] | ServCellIndex of NR Cell 1 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.7.1.1.3.3-7: RLC-BearerConfig-2 (Table 8.1.5.7.1.1.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-148 with Condition AM |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-BearerConfig ::= SEQUENCE { |  |  |  |
| logicalChannelIdentity | LogicalChannelIdentity with condition DRB j+1 | To ensure ID of secondary logical channel ID is different with existing logical channel and the primary logical channel defined in Table 8.1.5.7.1.1.3.3-6 |  |
| servedRadioBearer CHOICE { |  |  |  |
| drb-Identity | DRB-Identity with condition DRB j |  |  |
| } |  |  |  |
| rlc-Config CHOICE { |  |  |  |
| am SEQUENCE { |  |  |  |
| ul-AM-RLC SEQUENCE { |  |  |  |
| maxRetxThreshold | t1 | To ensure RLC failure happens before RLF |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| mac-LogicalChannelConfig SEQUENCE { |  |  |  |
| ul-SpecificParameters SEQUENCE { |  |  |  |
| allowedServingCells SEQUENCE (SIZE (1..maxNrofServingCells-1)) OF ServCellIndex { | 1 entry |  |  |
| ServCellIndex[1] | ServCellIndex of NR Cell 3 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.7.1.1.3.3-8: FailureInformation (Step 10, Table 8.1.5.7.1.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FailureInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| failureInformation SEQUENCE { |  |  |  |
| failureInfoRLC-Bearer SEQUENCE { |  |  |  |
| cellGroupId | CellGroupId as per TS 38.508-1 [4] Table 4.6.3-20 |  |  |
| logicalChannelIdentity | LogicalChannelIdentity with condition DRB j+1 |  |  |
| failureType | rlc-failure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.7.1.1.3.3-9: RRCReconfiguration (step 11, Table 8.1.5.7.1.1.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| masterCellGroup | CellGroupConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.7.1.1.3.3-10: CellGroupConfig (Table 8.1.5.7.1.1.3.3-9)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-19 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| sCellToAddModList | Not present |  |  |
| sCellToReleaseList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellIndex { | 1 entry |  |  |
| SCellIndex[1] | 1 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |

###### 8.1.5.7.1.2	Failure information / RLC failure / MCG / Inter-band CA

8.1.5.7.1.2.1	Test Purpose (TP)

Same as TC 8.1.5.7.1.1 but applied to Inter-band CA case.

8.1.5.7.1.2.2	Conformance requirements

Same as TC 8.1.5.7.1.1 but applied to Inter-band CA case.

8.1.5.7.1.2.3	Test description

8.1.5.7.1.2.3.1	Pre-test conditions

Same as test case 8.1.5.7.1.1 with the following differences:

-	CA configuration: Inter-band CA replaces Intra-band Contiguous CA.

-	Cells configuration: NR Cell 10 replaces NR Cell 3.

8.1.5.7.1.2.3.2	Test procedure sequence

Same as test case 8.1.5.7.1.1 with the following differences:

-	CA configuration: Inter-band CA replaces Intra-band Contiguous CA.

-	Cells configuration: NR Cell 10 replaces NR Cell 3.

8.1.5.7.1.2.3.3	Specific message contents

Same as test case 8.1.5.7.1.1 with the following differences.

-	CA configuration: Inter-band CA replaces Intra-band Contiguous CA.

-	Cells configuration: NR Cell 10 replaces NR Cell 3.

###### 8.1.5.7.1.3	Failure information / RLC failure / MCG / Intra-band non Contiguous CA

8.1.5.7.1.3.1	Test Purpose (TP)

Same as TC 8.1.5.7.1.1 but applied to Intra-band non Contiguous CA case.

8.1.5.7.1.3.2	Conformance requirements

Same as TC 8.1.5.7.1.1 but applied to Intra-band non Contiguous CA case.

8.1.5.7.1.3.3	Test description

8.1.5.7.1.3.3.1	Pre-test conditions

Same as test case 8.1.5.7.1.1 with the following differences:

-	CA configuration: Intra-band non Contiguous CA replaces Intra-band Contiguous CA.

8.1.5.7.1.3.3.2	Test procedure sequence

Same as test case 8.1.5.7.1.1 with the following differences:

-	CA configuration: Intra-band non Contiguous CA replaces Intra-band Contiguous CA.

8.1.5.7.1.3.3.3	Specific message contents

Same as test case 8.1.5.7.1.1 but applied to Intra-band non Contiguous CA case.

#### 8.1.5.8	Processing delay

##### 8.1.5.8.1	Processing delay / RRC_Idle to RRC_Connected / RRC_Inactive to RRC_Connected / Success / Latency check

8.1.5.8.1.1	Test Purpose (TP)

(1)

with { UE in NR RRC_IDLE state and has sent an RRCSetupRequest message }

ensure that {

when { UE receives RRCSetup message and after 10ms receives an UL grant }

then { UE successfully transmits RRCSetupComplete message }

}

(2)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE receives a SecurityModeCommand message and after 5ms receives an UL grant }

then { UE successfully transmits SecurityModeComplete }

}

(3)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE receives a RRCReconfiguration message to establish DRB that is not part of the current UE configuration and after 10ms receives an UL grant }

then { UE successfully transmits RRCReconfigurationComplete message }

}

(4)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE receives an UECapabilityEnquiry message and after 80ms receives an UL grant }

then { UE successfully transmits an UECapabilityInformation message }

}

(5)

with { UE in NR RRC_INACTIVE state and has sent an RRCResumeRequest message }

ensure that {

when { UE receives RRCResume message and after 6 or 10ms receives an UL grant depending on UE capability }

then { UE successfully transmits RRCResumeComplete message }

}

(6)

Void

(7)

with { UE in NR RRC_CONNECTED state and has sent an RRCReestablishmentRequest message after detecting radio link failure on expiring of timer T310 }

ensure that {

when { UE receives RRCReestablishment message and after 10ms receives an UL grant }

then { UE successfully transmits RRCReestablishmentComplete message }

}

(8)

Void

(9)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE receives CounterCheck message and after 5ms receives an UL grant }

then { UE successfully transmits an CounterCheckResponse message }

}

8.1.5.8.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331, clauses 5.3.3.4, 5.3.4.3, 5.3.5.3, 5.3.6.3, 5.3.7.4, 5.3.13.4, 5.6.1.3 and 12. Unless otherwise stated these are Rel-15 requirements.

[TS 38.331, clause 5.3.3.4]

The UE shall perform the following actions upon reception of the RRCSetup:

…

1>	submit the RRCSetupComplete message to lower layers for transmission, upon which the procedure ends.

[TS 38.331, clause 5.3.4.3]

The UE shall:

…

1>	if the SecurityModeCommand message passes the integrity protection check:

…

2>	submit the SecurityModeComplete message to lower layers for transmission, upon which the procedure ends;

[TS 38.331, clause 5.3.5.3]

The UE shall perform the following actions upon reception of the RRCReconfiguration:

…

1>	if the RRCReconfiguration includes the masterCellGroup:

2>	perform the cell group configuration for the received masterCellGroup according to 5.3.5.5;

…

1>	if the RRCReconfiguration message includes the radioBearerConfig:

2>	perform the radio bearer configuration according to 5.3.5.6;

…

1>	else (RRCReconfiguration was received via SRB1):

2>	submit the RRCReconfigurationComplete message via SRB1 to lower layers for transmission using the new configuration;

…

[38.331, clause 5.3.6.3]

Upon receiving the CounterCheck message, the UE shall:

…

1>	submit the CounterCheckResponse message to lower layers for transmission upon which the procedure ends.

[38.331, clause 5.3.7.4]

The UE shall set the contents of RRCReestablishmentRequest message as follows:

…

1>	submit the RRCReestablishmentRequest message to lower layers for transmission.

[38.331, clause 5.3.13.4]

The UE shall:

…

1>	if the RRCResume includes the masterCellGroup:

2>	perform the cell group configuration for the received masterCellGroup according to 5.3.5.5;

…

1>	submit the RRCResumeComplete message to lower layers for transmission;

…

[38.331, clause 5.6.1.3]

The UE shall set the contents of UECapabilityInformation message as follows:

1>	if the ue-CapabilityRAT-RequestList contains a UE-CapabilityRAT-Request with rat-Type set to nr:

2>	include in the ue-CapabilityRAT-ContainerList a UE-CapabilityRAT-Container of the type UE-NR-Capability and with the rat-Type set to nr;

2>	include the supportedBandCombinationList, featureSets and featureSetCombinations as specified in clause 5.6.1.4;

…

1>	submit the UECapabilityInformation message to lower layers for transmission, upon which the procedure ends.

[TS 38.331, clause 12]

The UE performance requirements for RRC procedures are specified in the following tables. The performance requirement is expressed as the time in [ms] from the end of reception of the network -> UE message on the UE physical layer up to when the UE shall be ready for the reception of uplink grant for the UE -> network response message with no access delay other than the TTI-alignment (e.g. excluding delays caused by scheduling, the random access procedure or physical layer synchronisation). In case the RRC procedure triggers BWP switching, the RRC procedure delay is the value defined in the following table plus the BWP switching delay defined in TS 38.133 [14], clause 8.6.3.

Figure 12.1-1: Illustration of RRC procedure delay

Table 12.1-1: UE performance requirements for RRC procedures for UEs

| Procedure title: | Network -> UE | UE -> Network | Value [ms] | Notes |
| --- | --- | --- | --- | --- |
| RRC Connection Control Procedures |  |  |  |  |
| RRC reconfiguration | RRCReconfiguration | RRCReconfigurationComplete | 10 |  |
| RRC reconfiguration (scell addition/release) | RRCReconfiguration | RRCReconfigurationComplete | 16 |  |
| RRC setup | RRCSetup | RRCSetupComplete | 10 |  |
| RRC re-establishment | RRCReestablishment | RRCReestablishmentComplete | 10 |  |
| RRC resume | RRCResume | RRCResumeComplete | 6 or 10 | Value=6 applies for a UE supporting reduced CP latency for the case of RRCResume message only including MAC and PHY configuration, reestablishPDCP and reestablishRLC for SRB2 and DRB(s), and no DRX, SPS, configured grant, CA or MIMO re-configuration will be triggered by this message. Further, the UL grant for transmission of RRCResumeComplete and the data is transmitted over common search space with DCI format 0_0. In this scenario, the RRC procedure delay [ms] can extend beyond the reception of the UL grant, up to 7 ms. For other cases, Value = 10 applies. |
| RRC resume (scell addition) | RRCResume | RRCResumeComplete | 16 |  |
| Initial AS security activation | SecurityModeCommand | SecurityModeComplete/SecurityModeFailure | 5 |  |
| Other procedures |  |  |  |  |
| UE capability transfer | UECapabilityEnquiry | UECapabilityInformation | 80 |  |
| Counter check | CounterCheck | CounterCheckResponse | 5 |  |

8.1.5.8.1.3	Test description

8.1.5.8.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 and NR Cell 2.

-	System information combination NR-2 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used in NR cells with RACH-ConfigGeneric in SIB1 set as per Table 8.1.5.8.1.3.3-0.

-	Test frequency NRf1 is as specified in TS 38.508-1 [4] clause 4.3.1 using the common highest UL and DL channel bandwidth and using the default subcarrier spacing specified in TS 38.508-1 [4] clause 6.2.3.1.

NOTE:	If pc_supportOfRedCap_r17=true OR pc_supportOfERedCap_r18=true, the FR1 test channel bandwidth is MIN {20MHz, common highest mandatory UL and DL channel bandwidth in clause 4.3.1 of TS 38.508-1 [4]} and If pc_supportOfRedCap_r17=true, the FR2 test channel bandwidth is MIN {100MHz, common highest mandatory UL and DL channel bandwidth in clause 4.3.1 of TS 38.508-1 [4]}

UE:

-	None.

Preamble:

-	The UE is in state RRC_IDLE using generic procedure parameter Connectivity (NR) according to TS 38.508-1 [4], clause 4.5.2.

8.1.5.8.1.3.2	Test procedure sequence

Table 8.1.5.8.1.3.2-1/2 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while column marked "T1", is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.5.8.1.3.2-1: Time instances of cell power level and parameter changes in FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 |
| --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -96 |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | “Off” | -88 |

Table 8.1.5.8.1.3.2-2: Time instances of cell power level and parameter changes in FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 |
| --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -91 |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | “Off” | -82 |

Table 8.1.5.8.1.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a Paging message including a matched ng-5G-S-TMSI. | <-- | NR RRC: Paging | - | - |
| 2 | The UE transmits an RRCSetupRequest message. | --> | NR RRC: RRCSetupRequest | - | - |
| 3 | The SS transmits an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 3A | The SS starts transmitting a periodic DCI from (10∙2μ+1+TDL) slots after step 3 to schedule PUSCH. (Note 1, Note 2) | - | - | - | - |
| 4 | Check: Does the UE transmit an RRCSetupComplete message piggybacked with SERVICE REQUEST message within (10∙2μ+1+TDL+K2) slots after successful completion of step 3? (Note 1, Note 2, Note 3) | --> | NR RRC: RRCSetupComplete NR 5GMM: SERVICE REQUEST | 1 | P |
| 5 | The SS transmits a SecurityModeCommand message. | <-- | NR RRC: SecurityModeCommand | - | - |
| 5A | The SS starts transmitting a periodic DCI from (5∙2μ+1+TDL) slots after step 5 to schedule PUSCH. (Note 1, Note 2) | - | - | - | - |
| 6 | Check: Does the UE transmit a SecurityModeComplete message within (5∙2μ+1+TDL+K2) slots after successful completion of step 5? (Note 1, Note 2, Note 3) | --> | NR RRC: SecurityModeComplete | 2 | P |
| 7 | The SS transmits a UECapabilityEnquiry message. | <-- | NR RRC: UECapabilityEnquiry | - | - |
| 7A | The SS starts transmitting a periodic DCI with the maximum grant allowed by the carrier bandwidth with MCS=24 from (80∙2μ+1+TDL) slots after step 7 to schedule PUSCH. (Note 1, Note 2) | - | - | - | - |
| 8 | Check: Does the UE transmit a UECapabilityInformation message within (80∙2μ+1+TDL+K2) slots after successful completion of step 7? (Note 1, Note 2, Note 3) | --> | NR RRC: UECapabilityInformation | 4 | P |
| 9 | The SS transmits an RRCReconfiguration message to establish SRB2 and DRB #n for the UE piggybacked with SERVICE ACCEPT message. | <-- | NR RRC: RRCReconfiguration NR 5GMM: SERVICE ACCEPT | - | - |
| 9A | The SS starts transmitting a periodic DCI from (10∙2μ+1+TDL) slots after step 9 to schedule PUSCH. (Note 1, Note 2) | - | - | - | - |
| 10 | Check: Does the UE transmit an RRCReconfigurationComplete message within (10∙2μ+1+TDL+K2) slots after successful completion of step 9? (Note 1, Note 2, Note 3) | --> | NR RRC: RRCReconfigurationComplete | 3 | P |
| 11 | The SS transmits a CounterCheck message. | <-- | NR RRC: CounterCheck | - | - |
| 11A | The SS starts transmitting a periodic DCI from (5∙2μ+1+TDL) slots after step 11 to schedule PUSCH. (Note 1, Note 2) | - | - | - | - |
| 12 | Check: Does the UE transmit a CounterCheckResponse message within (5∙2μ+1+TDL+K2) slots after successful completion of step 11? (Note 1, Note 2, Note 3) | --> | NR RRC: CounterCheckResponse | 9 | P |
| 13 | Void | - | - | - | - |
| - | EXCEPTION: Steps 14a1 to 14a7 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if a capability is supported. | - | - | - | - |
| 14a1 | IF pc_inactiveState THEN the SS transmits an RRCRelease message with suspendConfig. | <-- | NR RRC: RRCRelease | - | - |
| 14a2 | Wait 1s to let UE enter RRC_INACTIVE state. | - | - | - | - |
| 14a3 | The SS transmits a Paging message including a matched ng-5G-S-TMSI. | <-- | NR RRC: Paging | - | - |
| 14a4 | The UE transmits an RRCResumeRequest message. | --> | NR RRC: RRCResumeRequest | - | - |
| 14a5 | The SS transmits an RRCResume message for the UE on slot #i in the radio frame: For μ = 0, if FD-FDD and pc_reducedCP_Latency then i = 5 else i = 0. For μ = 1, i = 1 For μ = 3, if pc_reducedCP_Latency then I = 41 else i = 21 (Note 2) | <-- | NR RRC: RRCResume | - | - |
| - | EXCEPTION: Steps 14a5Aa1 to 14a5Ab2 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if a capability is supported. | - | - | - | - |
| 14a5Aa1 | IF pc_reducedCP_Latency THEN the SS starts transmitting a periodic DCI from (6∙2μ+1+TDL) slots after step 14a5 to schedule PUSCH. IF UE transmits a Random Access Preamble, then SS sends RAR. (Note 1, Note 2) | - | - | - | - |
| 14a5Aa2 | Check: Does the UE transmit an RRCResumeComplete message within (6∙2μ+1+TDL+K2) slots after successful completion of step 14a5? (Note 1, Note 2, Note 3) | --> | NR RRC: RRCResumeComplete | 5 | P |
| 14a5Ab1 | ELSE the SS starts transmitting a periodic DCI from (10∙2μ+1+TDL) slots after step 14a5 to schedule PUSCH. IF UE transmits a Random Access Preamble, then SS sends RAR. (Note 1, Note 2) | - | - | - | - |
| 14a5Ab2 | Check: Does the UE transmit an RRCResumeComplete message within (10∙2μ+1+TDL+K2) slots after successful completion of step 14a5? (Note 1, Note 2, Note 3) | --> | NR RRC: RRCResumeComplete | 5 | P |
| 14a6-14a7 | Void | - | - | - | - |
| 14-21 | Void | - | - | - | - |
| 22 | The SS re-adjusts the SS levels according to row "T1" in table 8.1.5.8.1.3.2-1/2 in order that the radio link quality of NR Cell 1 is degraded, and NR Cell 2 is suitable for camping. | - | - | - | - |
| 23 | Wait 1s to let T310 expire. | - | - | - | - |
| 24 | The UE transmits an RRCReestablishmentRequest message on Cell 2. | --> | NR RRC: RRCReestablishmentRequest | - | - |
| 25 | The SS transmits an RRCReestablishment message on slot #i in the radio frame, where: i = 1 for μ=0,1 i = 21 for μ=3 (Note 2) | <-- | NR RRC: RRCReestablishment | - | - |
| 25A | The SS starts transmitting a periodic DCI from (10∙2μ+1+TDL) slots after step 25 to schedule PUSCH. IF UE transmits a Random Access Preamble, then SS sends RAR. (Note 1, Note 2) | - | - | - | - |
| 26 | Check: Does the UE transmit an RRCReestablishmentComplete message within (10∙2μ+1+TDL+K2) slots after successful completion of step 25? (Note 1, Note 2, Note 3) | --> | NR RRC: RRCReestablishmentComplete | 7 | P |
| 26A | The SS transmits an RRCReconfiguration message to resume existing radio bearer. | <-- | NR RRC: RRCReconfiguration | - | - |
| 26B | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationtComplete | - | - |
| 27-35 | Void | - | - | - | - |
| Note 1: TDL is the delay in slots to the next available DL slot suitable for the transmission of a DCI. Note 2: μ is the SCS index. μ = 0/1/2/3 for SCS = 15kHz/30kHz/60kHz/120kHz respectively. Note 3: K2 is the offset (in slots) between UL grant and the PUSCH it scheduled. Note 4: If pc_supportOfERedCap_r18=true, the SS configure the grant with the LRB = 7 and MCS= 24 to ensure the TBsize less than 5000 bits. |  |  |  |  |  |

8.1.5.8.1.3.3	Specific message contents

Table 8.1.5.8.1.3.3-0: RACH-ConfigGeneric (Preamble and all steps)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| ra-ResponseWindow | sl20 |  |  |
|  | sl4 |  | FR1 AND FDD AND NOT(pc_halfDuplexFDD_TypeA_RedCap_r17) |
|  | sl10 |  | FR1 AND (TDD OR pc_halfDuplexFDD_TypeA_RedCap_r17) AND SCS15 |
| } |  |  |  |

Table 8.1.5.8.1.3.3-1: RRCReconfiguration (Step 9, Table 8.1.5.8.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], table 4.6.1-13 with condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig-SRB2-DRB(1,0) | TS 38.508-1 [4] Table 4.8.1-4 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig-SRB2-DRB(1,0) | TS 38.508-1 [4] Table 4.8.1-2B |  |
| dedicatedNAS-MessageList | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.8.1.3.3-2: CounterCheck (Step 11, Table 8.1.5.8.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CounterCheck ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| counterCheck SEQUENCE { |  |  |  |
| drb-CountMSB-InfoList SEQUENCE (SIZE (1..maxDRB)) OF DRB-CountMSB-Info { | 1 entry |  |  |
| DRB-CountMSB-Info [1] SEQUENCE { |  |  |  |
| drb-Identity | DRB-Identity with Condition DRBn | entry 1 n is the index of the DRB established in step 9 of Table 8.1.5.8.1.3.2-3, which is allocated according to internal TTCN mapping |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.8.1.3.3-3: CounterCheckResponse (Step 12, Table 8.1.5.8.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CounterCheckResponse ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| counterCheckResponse SEQUENCE { |  |  |  |
| drb-CountInfoList SEQUENCE (SIZE (0..maxDRB)) OF DRB-CountInfo { | 1 entry |  |  |
| DRB-CountInfo[1] SEQUENCE { |  |  |  |
| drb-Identity | DRB-Identity with Condition DRBn | entry 1 n is the index of the DRB established in step 9 of Table 8.1.5.8.1.3.2-3, which is allocated according to internal TTCN mapping |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.8.1.3.3-4: RRCRelease (Step 14a1, Table 8.1.5.8.1.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-16 with Condition NR_RRC_INACTIVE

Table 8.1.5.8.1.3.3-5 to Table 8.1.5.8.1.3.3-10: Void

Table 8.1.5.8.1.3.3-11: Physical layer parameters for DCI format 1_0 (Steps 3, 14a5 and 25, Table 8.1.5.8.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.3.6.1.2.1-1 |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| PDSCH-to-HARQ_feedback timing indicator | K1 slots as specified in 9.2.3 in TS 38.213 |  |  |
|  | µ=0 (SCS=15kHz): K1=3 | '010'B |  |
|  | µ=1 (SCS=30kHz): K1=7 | '110'B |  |
|  | µ=3 (SCS=120kHz): K1=3 | '010'B |  |

Table 8.1.5.8.1.3.3-12: SIB1 (Preamble, and all steps of Table 8.1.5.8.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| servingCellConfigCommon SEQUENCE { |  |  |  |
| uplinkConfigCommon SEQUENCE { |  |  |  |
| initialUplinkBWP SEQUENCE { |  |  |  |
| rach-ConfigCommon CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| rach-ConfigGeneric SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 159 |  | FR2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.8.1.3.3-13: RRCReconfiguration (Step 26A, Table 8.1.5.8.1.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition REEST

##### 8.1.5.8.2	Processing delay / RRC_Inactive to RRC_Connected / Success / Latency check / SCell addition

###### 8.1.5.8.2.1	Processing delay / RRC_Inactive to RRC_Connected / Success / Latency check / SCell addition / Intra-band Contiguous CA

8.1.5.8.2.1.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE receives an RRCReconfiguration message containing sCellToAddModList with a SCell addition and after 16ms receives an UL grant }

then { UE successfully transmits RRCReconfigurationComplete message }

}

(2)

with { UE in NR RRC_INACTIVE state and has sent an RRCResumeRequest message }

ensure that {

when { UE receives RRCResume message containing sCellToAddModList with a SCell addition and after 16ms receives an UL grant }

then { UE successfully transmits RRCResumeComplete message }

}

8.1.5.8.2.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331, clauses 5.3.3.4, 5.3.4.3, 5.3.5.3, 5.3.6.3, 5.3.7.4, 5.3.13.4, 5.6.1.3 and 12. Unless otherwise stated these are Rel-15 requirements.

[TS 38.331, clause 5.3.5.3]

The UE shall perform the following actions upon reception of the RRCReconfiguration:

…

1>	if the RRCReconfiguration includes the masterCellGroup:

2>	perform the cell group configuration for the received masterCellGroup according to 5.3.5.5;

…

1>	if the RRCReconfiguration message includes the radioBearerConfig:

2>	perform the radio bearer configuration according to 5.3.5.6;

…

1>	else (RRCReconfiguration was received via SRB1):

2>	submit the RRCReconfigurationComplete message via SRB1 to lower layers for transmission using the new configuration;

…

[38.331, clause 5.3.13.4]

The UE shall:

…

1>	if the RRCResume includes the masterCellGroup:

2>	perform the cell group configuration for the received masterCellGroup according to 5.3.5.5;

…

1>	submit the RRCResumeComplete message to lower layers for transmission;

…

[TS 38.331, clause 12]

The UE performance requirements for RRC procedures are specified in the following tables. The performance requirement is expressed as the time in [ms] from the end of reception of the network -> UE message on the UE physical layer up to when the UE shall be ready for the reception of uplink grant for the UE -> network response message with no access delay other than the TTI-alignment (e.g. excluding delays caused by scheduling, the random access procedure or physical layer synchronisation). In case the RRC procedure triggers BWP switching, the RRC procedure delay is the value defined in the following table plus the BWP switching delay defined in TS 38.133 [14], clause 8.6.3.

Figure 12.1-1: Illustration of RRC procedure delay

Table 12.1-1: UE performance requirements for RRC procedures for UEs

| Procedure title: | Network -> UE | UE -> Network | Value [ms] | Notes |
| --- | --- | --- | --- | --- |
| RRC Connection Control Procedures |  |  |  |  |
| RRC reconfiguration (scell addition/release) | RRCReconfiguration | RRCReconfigurationComplete | 16 |  |
| RRC resume (scell addition) | RRCResume | RRCResumeComplete | 16 |  |

8.1.5.8.2.1.3	Test description

8.1.5.8.2.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the PCell and NR Cell 3 is the SCell.

-	System information combination NR-4 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used in NR cells.

UE:

-	None.

Preamble:

-	The UE is in 5GS state 3N-A on NR Cell 1 according to TS 38.508-1 [4], clause 4.5.4.

8.1.5.8.2.1.3.2	Test procedure sequence

Table 8.1.5.8.2.1.3.2-1/2 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble.

Table 8.1.5.8.2.1.3.2-1: Time instances of cell power level and parameter changes in FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 |
| --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -88 |

Table 8.1.5.8.2.1.3.2-2: Time instances of cell power level and parameter changes in FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 |
| --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -82 |

Table 8.1.5.8.2.1.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message with sCellToAddModList to add SCell (Cell 3). | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The SS starts transmitting a periodic DCI from (16∙2μ+1+TDL) slots after step 1 to schedule PUSCH. (Note 1, Note 2) | - | - | - | - |
| 3 | Check: Does the UE transmit an RRCReconfigurationComplete message within (16∙2μ+1+TDL+K2) slots after successful completion of step 1? (Note 1, Note 2, Note 3) | --> | NR RRC: RRCReconfigurationComplete | 1 | P |
| 4 | The SS transmits an RRCReconfiguration message to release SCell (Cell 3). | <-- | NR RRC: RRCReconfiguration | - | - |
| 5 | UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| - | EXCEPTION: Steps 6a1 to 6a9 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that takes place if a capability is configured. | - | - | - | - |
| 6a1 | IF pc_inactiveState THEN the SS transmits an RRCRelease message with suspendConfig. | <-- | NR RRC: RRCRelease | - | - |
| 6a2 | Wait 1s to let UE enter RRC_INACTIVE state. | - | - | - | - |
| 6a3 | The SS transmits a Paging message including a matched ng-5G-S-TMSI. | <-- | NR RRC: Paging | - | - |
| 6a4 | The UE transmits an RRCResumeRequest message. | --> | NR RRC: RRCResumeRequest | - | - |
| 6a5 | The SS transmits an RRCResume message with sCellToAddModList to add SCell (Cell 3) on slot #i in the radio frame, where: i = 1 for μ=0,1 i = 21 for μ=3 (Note 2) | <-- | NR RRC: RRCResume | - | - |
| 6a6 | The SS starts transmitting a periodic DCI from (16∙2μ+1+TDL) slots after step 6a5 to schedule PUSCH. IF UE transmits a Random Access Preamble, then SS sends RAR.(Note 1, Note 2) | - | - | - | - |
| 6a7 | Check: Does the UE transmit an RRCResumeComplete message within (16∙2μ+1+TDL+K2) slots after successful completion of step 6a5? (Note 1, Note 2, Note 3) | --> | NR RRC: RRCResumeComplete | 2 | P |
| 6a8 | The SS transmits an RRCReconfiguration message containing an sCellToReleaseList with SCell NR Cell 3. | <-- | NR RRC: RRCReconfiguration | - | - |
| 6a9 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationtComplete | - | - |
| Note 1: TDL is the delay in slots to the next available DL slot suitable for the transmission of a DCI. Note 2: μ is the SCS index. μ = 0/1/2/3 for SCS = 15kHz/30kHz/60kHz/120kHz respectively. Note 3: K2 is the offset (in slots) between DCI and the PUSCH it scheduled. |  |  |  |  |  |

8.1.5.8.2.1.3.3	Specific message contents

Table 8.1.5.8.2.1.3.3-1: RRCReconfiguration (Step 1, Table 8.1.5.8.2.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with Condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig | Table 8.1.5.8.2.1.3.3-2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.8.2.1.3.3-2: CellGroupConfig (Table 8.1.5.8.2.1.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with Condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig { | 1 entry |  |  |
| SCellConfig[1] SEQUENCE { |  | entry 1 |  |
| sCellConfigCommon SEQUENCE { |  |  |  |
| physCellId | physCellId of Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.8.2.1.3.3-3: RRCReconfiguration (Steps 4 and 6a8, Table 8.1.5.8.2.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with Condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup SEQUENCE { |  |  |  |
| sCellToAddModList | Not present |  |  |
| sCellToReleaseList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellIndex { | 1 entry |  |  |
| SCellIndex[1] | SCellIndex of Cell 3 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.8.2.1.3.3-4: RRCRelease (Step 6a1, Table 8.1.5.8.2.1.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-16 with Condition NR_RRC_INACTIVE

Table 8.1.5.8.2.1.3.3-5: RRCResume (Step 6a5, Table 8.1.5.8.2.1.3.2-3)

| Derivation Path: TS 38.508-1[4],Table 4.6.1-17 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCResume ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcResume SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig | Table 8.1.5.8.2.1.3.3-6 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.8.2.1.3.3-6: CellGroupConfig (Table 8.1.5.8.2.1.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with Condition SCell_add and RESUME |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig { | 1 entry |  |  |
| SCellConfig[1] SEQUENCE { |  | entry 1 |  |
| sCellConfigCommon SEQUENCE { |  |  |  |
| physCellId | physCellId of Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.8.2.1.3.3-7: Physical layer parameters for DCI format 1_0 (Steps 6a5, Table 8.1.5.8.2.1.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.3.6.1.2.1-1 |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| PDSCH-to-HARQ_feedback timing indicator | K1 slots as specified in 9.2.3 in TS 38.213 |  |  |
|  | µ=0 (SCS=15kHz): K1=3 | '010'B |  |
|  | µ=1 (SCS=30kHz): K1=7 | '110'B |  |
|  | µ=3 (SCS=120kHz): K1=3 | '010'B |  |

###### 8.1.5.8.2.2	Processing delay / RRC_Inactive to RRC_Connected / Success / Latency check / SCell addition / Inter-band CA

8.1.5.8.2.2.1	Test Purpose (TP)

Same as TC 8.1.5.8.2.1 but applied to Inter-band CA Test Case

8.1.5.8.2.2.2	Conformance requirements

Same as TC 8.1.5.8.2.1 but applied to Inter-band CA Test Case

8.1.5.8.2.2.3	Test description

8.1.5.8.2.2.3.1	Pre-test conditions

Same as TC 8.1.5.8.2.1 but with the following differences：

-	CA configuration: Inter-band CA replaces Intra-band Contiguous CA.

-	Cells configuration: Cell 10 replaces Cell 3.

8.1.5.8.2.2.3.2	Test procedure sequence

Same as TC 8.1.5.8.2.1 but with the following differences：

-	Cells configuration: Cell 10 replaces Cell 3.

8.1.5.8.2.2.3.3	Specific message contents

Same as TC 8.1.5.8.2.1 but with the following differences：

-	Cells configuration: Cell 10 replaces Cell 3.

###### 8.1.5.8.2.3	Processing delay / RRC_Inactive to RRC_Connected / Success / Latency check / SCell addition / Intra-band non-Contiguous CA

8.1.5.8.2.3.1	Test Purpose (TP)

Same as TC 8.1.5.8.2.1 but applied to Intra-band non-Contiguous CA Test Case

8.1.5.8.2.3.2	Conformance requirements

Same as TC 8.1.5.8.2.1 but applied to Intra-band non-Contiguous CA Test Case

8.1.5.8.2.3.3	Test description

8.1.5.8.2.3.3.1	Pre-test conditions

Same as TC 8.1.5.8.2.1 but with the following differences:

-	CA configuration: Intra-band non-Contiguous CA replaces Intra-band Contiguous CA.

8.1.5.8.2.3.3.2	Test procedure sequence

Same as TC 8.1.5.8.2.1.

8.1.5.8.2.3.3.3	Specific message contents

Same as TC 8.1.5.8.2.1.

#### 8.1.5.9	Message Segment transfer

##### 8.1.5.9.1	UECapabilityInformation / UL segment transfer

8.1.5.9.1.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE receives UECapabilityEnquiry message with rrc-segAllowed-r16 enabled }

then { UE successfully segments UECapabilityInformation message at RRC layer and sends UlDedicatedMessageSegment messages with rrc-MessageSegmentType set accordingly for the first and last segment and rrc-MessageSegmentContainer containing the segments of UECapabilityInformation message }

}

(2)

with { UE in NR RRC_CONNECTED state and UE having received UECapabilityEnquiry message with rrc-segAllowed-r16 enabled and having transmitted the firstSegment of UECapabilityInformation message within the ULDedicatedMessageSegment message }

ensure that {

when { UE detects radio link failure on expiry of timer T310 }

then { UE successfully completes RRC re-establishment procedure and discards the remaining segments of UECapabilityInformation message }

}

8.1.5.9.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331 clause 5.6.1.3, 5.7.7.2, 5.7.7.3 and 5.3.7.2. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.6.1.3]

The UE shall set the contents of UECapabilityInformation message as follows:

1>	if the RRC message segmentation is enabled based on the field rrc-SegAllowed received, and the encoded RRC message is larger than the maximum supported size of a PDCP SDU specified in TS 38.323 [5]:

2>	initiate the UL message segment transfer procedure as specified in clause 5.7.7;

1>	else:

2>	submit the UECapabilityInformation message to lower layers for transmission, upon which the procedure ends.

[TS 38.331, clause 5.7.7.2]

A UE capable of UL RRC message segmentation in RRC_CONNECTED will initiate the procedure when the following conditions are met:

1>	if the RRC message segmentation is enabled based on the field rrc-SegAllowed received, and

1>	if the encoded RRC message is larger than the maximum supported size of a PDCP SDU specified in TS 38.323 [5];

Upon initiating the procedure, the UE shall:

1>	initiate transmission of the ULDedicatedMessageSegment message as specified in 5.7.7.3;

[TS 38.331, clause 5.7.7.3]

The UE shall segment the encoded RRC PDU based on the maximum supported size of a PDCP SDU specified in TS 38.323 [5]. UE shall minimize the number of segments and set the contents of the ULDedicatedMessageSegment messages as follows:

1>	F or each new UL DCCH message, set the segmentNumber to 0 for the first message segment and increment the segmentNumber for each subsequent RRC message segment;

1>	set rrc-MessageSegmentContainer to include the segment of the UL DCCH message corresponding to the segmentNumber;

1>	if the segment included in the rrc-MessageSegmentContainer is the last segment of the UL DCCH message:

2>	set the rrc-MessageSegmentType to lastSegment;

1>	else:

2>	set the rrc-MessageSegmentType to notLastSegment;

1>	submit all the ULDedicatedMessageSegment messages generated for the segmented RRC message to lower layers for transmission in ascending order based on the segmentNumber, upon which the procedure ends.

[TS 38.331, clause 5.3.7.2]

Upon initiation of the procedure, the UE shall:

…

1>	if UE is not configured with conditionalReconfiguration:

2>	reset MAC;

2>	release spCellConfig, if configured;

2>	suspend all RBs, except SRB0;

8.1.5.9.1.3	Test description

8.1.5.9.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

UE:

-	None.

Preamble:

-	The UE is in state 3N-A as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1.

8.1.5.9.1.3.2	Test procedure sequence

Table 8.1.5.9.1.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 |
| --- | --- | --- | --- |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | “Off” |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -88 |
| Power level “Off” is defined in TS 38.508-1 [4] Table 6.2.2.1-3. |  |  |  |

Table 8.1.5.9.1.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 |
| --- | --- | --- | --- |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | “Off” |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -82 |
| Power level “Off” is defined in TS 38.508-1 [4] Table 6.2.2.2-2. |  |  |  |

Table 8.1.5.9.1.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a UECapabilityEnquiry message to enquire max capability and with rrc-segAllowed-r16 set to enabled. | <-- | NR RRC: UECapabilityEnquiry | - | - |
| - | EXCEPTION: Steps 2a1 to 2b7 describe behaviour that depends on the size of UEs capability container; the "lower case letter" identifies a step sequence that takes place depending on the size of UE capability message. IF UE’s encoded UECapabilityInformation message >= Max PDCP SDU size then step 2a1 will be performed else step 2b1 to 2b7 will be performed. | - | - | - | - |
| - | EXCEPTION: Step 2a1 is repeated a maximum of 16 times or till rrc-MessageSegmentType-16 IE within the ULDedicatedMessageSegment message indicates rrc-MessageSegmentType-r16 = lastSegment. | - | - | - | - |
| 2a1 | Check: Does the UE send segments of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment message? NOTE: The SS shall ensure that the concatenated segments decode into a valid UECapabilityInformation message (specific capability content is not checked). | --> | NR RRC: ULDedicatedMessageSegment | 1 | P |
| 2b1 | Check: Does the UE send UECapabilityInformation Message? | --> | NR RRC: UECapabilityInformation | - | - |
| 3a1-4a13 | Void | - | - | - | - |
| 2b2 | The SS transmits a SET UL MESSAGE REQUEST message. | <-- | SET UL MESSAGE REQUEST | - | - |
| 2b3 | The UE transmits a SET UL MESSAGE RESPONSE message. | --> | SET UL MESSAGE RESPONSE | - | - |
| 2b4 | The SS transmits a UECapabilityEnquiry message with rrc-segAllowed-r16 set to enabled. | <-- | NR RRC: UECapabilityEnquiry | - | - |
| - | EXCEPTION: Step 2b5 is repeated a maximum of 2 times. | - | - | - | - |
| 2b5 | Check: Does the UE send segments of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment message? NOTE: The SS shall ensure that the concatenated segments decode into a valid UECapabilityInformation message (specific capability content is not checked). | --> | NR RRC: ULDedicatedMessageSegment | 1 | P |
| 2b6 | The SS transmits a SET UL MESSAGE REQUEST message. | <-- | SET UL MESSAGE REQUEST | - | - |
| 2b7 | The UE transmits a SET UL MESSAGE RESPONSE message. | --> | SET UL MESSAGE RESPONSE | - | - |
| 3 | SS stops regular grant transmission upon scheduling request. | - | - | - | - |
| 4 | The SS transmits a UECapabilityEnquiry message to enquire max capability and with rrc-segAllowed-r16 set to enabled. | <-- | NR RRC: UECapabilityEnquiry | - | - |
| 5 | The SS allocates 15 UL grants from (80∙2μ+1+TDL) slots after step 4 at an interval of 10ms to schedule PUSCH till the ULDedicatedMessageSegment at step 6 is received. (Note 1, Note 2, Note 3) | - | - | - | - |
| 6 | The UE sends 1st segment of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment message. | --> | NR RRC: ULDedicatedMessageSegment | - | - |
| 7 | The SS changes NR Cell 1 parameters according to the row "T1" in table 8.1.5.9.1.3.2-1/2 in order that the radio link quality of NR Cell 1 is degraded. | - | - | - | - |
| 8 | Wait for T310 to expire | - | - | - | - |
| 9 | The SS changes NR Cell 1 parameters according to the row "T2" in table 8.1.5.9.1.3.2-1/2. | - | - | - | - |
| 10 | The UE sends an RRCReestablishmentRequest message on NR Cell 1. | --> | NR RRC: RRCReestablishmentRequest | - | - |
| 11 | The SS transmits an RRCReestablishment message to resume SRB1 operation on NR Cell 1. | <-- | NR RRC: RRCReestablishment | - | - |
| 12 | The UE transmits an RRCReestablishmentComplete message on NR Cell 1. | --> | NR RRC: RRCReestablishmentComplete | - | - |
| 12A | The SS transmits an RRCReconfiguration message to resume existing radio bearer. | <-- | NR RRC: RRCReconfiguration | - | - |
| 12B | The UE transmit an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationtComplete | - | - |
| 13 | SS resumes regular UL grant transmission. | - | - | - | - |
| 14 | Check: Upon successful completion of RRC re-establishment procedure, does the UE discards remaining segments of UECapabilityInformation message and does not send any ULDedicatedMessageSegment message in the next 5 seconds? | - | - | 2 | P |
| Note 1: TDL is the delay in slots to the next available DL slot suitable for the transmission of a DCI. Note 2: μ is the SCS index. μ = 0/1/2/3 for SCS = 15kHz/30kHz/60kHz/120kHz respectively. Note 3: 15 UL grants of default size (624 bytes, using LRBs & IMCS as per 38.523-3[3] Table 7.1.2.3.3-1) are chosen to enable UE to transmit only one ulDedicatedMessageSegment as maximum PDCP SDU size 9000 bytes. PDCP is segmented into several RLC Segments. Each MAC TB contains 3 bytes of MAC header + 2 bytes of Short BSR + 3 bytes of RLC Header + Payload of Segmented RLC SDU. Additional 8 bytes are required in each RLC segment. 15 grants provide (15*624=9360) which is sufficient to accommodate 9000 bytes and RLC/MAC header and MAC CE. |  |  |  |  |  |

8.1.5.9.1.3.3	Specific message contents

Table 8.1.5.9.1.3.3-0: CellGroupConfig (preamble)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlf-TimersAndConstants SEQUENCE { |  |  |  |
| t311-v1530 | ms10000 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.9.1.3.3-1: UECapabilityEnquiry (step 1, 2b4 and step 4, Table 8.1.5.9.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-31 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UECapabilityEnquiry ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueCapabilityEnquiry SEQUENCE { |  |  |  |
| ue-CapabilityRAT-RequestList SEQUENCE (SIZE (1.. maxRAT-CapabilityContainers)) OF UE-CapabilityRAT-Request { | 2 entries |  |  |
| UE-CapabilityRAT-Request[1] SEQUENCE { |  | entry 1 |  |
| rat-Type | nr |  |  |
| capabilityRequestFilter | UE-CapabilityRequestFilterNR | OCTET STRING (CONTAINING UE-CapabilityRequestFilterNR) |  |
| } |  |  |  |
| UE-CapabilityRAT-Request[2] SEQUENCE { |  | entry 2 |  |
| rat-Type | eutra-nr |  |  |
| capabilityRequestFilter | UE-CapabilityRequestFilterNR | OCTET STRING (CONTAINING UE-CapabilityRequestFilterNR) |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| ue-CapabilityEnquiryExt | OCTET STRING Containing UECapabilityEnquiry-v1560-IEs |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.9.1.3.3-2: UE-CapabilityRequestFilterNR (Table 8.1.5.9.1.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.4-47 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE-CapabilityRequestFilterNR ::= SEQUENCE { |  |  |  |
| frequencyBandListFilter | FreqBandList |  |  |
| } |  |  |  |

Table 8.1.5.9.1.3.3-3: FreqBandList (Table 8.1.5.9.1.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.4-21 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FreqBandList::= SEQUENCE (SIZE (1..maxBandsMRDC)) OF FreqBandInformation { | 43 entries | FreqBandIndicatorNR = n1, n2, n3, n5, n7, n8, n12, n14, n20, n25, n26, n28, n29, n30, n34, n38, n39, n40, n41, n48, n50, n51, n53, n65, n66, n70, n71, n74, n75, n76, n77, n78, n79, n80, n81, n82, n83, n84, n86, n257, n258, n260, n261. |  |
| FreqBandInformation[n] CHOICE { |  | entry n |  |
| bandInformationNR SEQUENCE { |  |  |  |
| bandNR | FreqBandIndicatorNR |  |  |
| maxBandwidthRequestedDL | Not present |  |  |
| maxBandwidthRequestedUL | Not present |  |  |
| maxCarriersRequestedDL | Not present |  |  |
| maxCarriersRequestedUL | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.9.1.3.3-4: UECapabilityEnquiry-v1560-IEs (Table 8.1.5.9.1.3.3-1)

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| UECapabilityEnquiry-v1560-IEs ::= SEQUENCE { |  |  |  |
| capabilityRequestFilterCommon | UE-CapabilityRequestFilterCommon |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rrc-SegAllowed-r16 | enabled |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.9.1.3.3-4A: UE-CapabilityRequestFilterCommon

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| UE-CapabilityRequestFilterCommon ::= SEQUENCE { |  |  |  |
| mrdc-Request SEQUENCE { |  |  |  |
| omitEN-DC | Not present |  |  |
| includeNR-DC | true |  |  |
| includeNE-DC | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.9.1.3.3-5: ULDedicatedMessageSegment (step 2a1, 2b5 and step 6, Table 8.1.5.9.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ULDedicatedMessageSegment-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| segmentNumber-r16 | 0 for first segment, and incremented by 1 for each subsequent segment |  |  |
| rrc-MessageSegmentContainer-r16 | Not Checked | OCTET STRING Includes a segment of the encoded UL DCCH message. The size of the included segment in this container should be small enough so the resulting encoded RRC message PDU is less than or equal to the PDCP SDU size limit |  |
| rrc-MessageSegmentType-r16 | notLastSegment for all segments except the last segment. lastSegment for the last segment |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.9.1.3.3-6: UECapabilityInformation (step 2b1, Table 8.1.5.9.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UECapabilityInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueCapabilityInformation SEQUENCE { |  |  |  |
| ue-CapabilityRAT-ContainerList SEQUENCE (SIZE (1..maxRAT-Capabilities)) OF UE-CapabilityRAT-Container { | 1 entry or 2 entries | 1 entry if EN-DC is not supported otherwise 2 entries |  |
| UE-CapabilityRAT- Container[1] SEQUENCE { |  | entry 1 |  |
| rat-Type | nr |  |  |
| ueCapabilityRAT-Container | UE-NR-Capability | Encoded as per TS 38.331 [12] clause 5.6.1 |  |
| } |  |  |  |
| UE-CapabilityRAT- Container[2] SEQUENCE { |  | entry 2 | EN-DC |
| rat-Type | eutra-nr |  |  |
| ueCapabilityRAT-Container | UE-MRDC-Capability | Encoded as per TS 38.331 [12] clause 5.6.1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.9.1.3.3-7: RRCReconfiguration (step 12A, Table 8.1.5.9.1.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition REEST

##### 8.1.5.9.2	RRC reconfiguration / DL segment transfer

8.1.5.9.2.1	Test Purpose (TP)

(1)

with { UE having completed the RRC connection establishment procedure}

ensure that {

when { SS initiates the DL Dedicated Message Segment transfer procedure to send an RRCReconfiguration message to establish a DRB and SRB2}

then { UE successfully completes the reconfiguration }

}

(2)

with { UE in NR RRC_CONNECTED state with SRB2 and has previously received the first and second  segment of an RRCReconfiguration message including SRB2 configuration }

ensure that {

when { SS releases the RRC connection }

then { UE discards all segments }

}

8.1.5.9.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in:  TS 38.331, clauses 5.3.11 and 5.7.6.3. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.3.11]

The UE shall:

...

1>	discard any segments of segmented RRC messages stored according to 5.7.6.3;

[TS 38.331, clause 5.7.6.3]

Upon receiving DLDedicatedMessageSegment message, the UE shall:

1>	store the segment included in rrc-MessageSegmentContainer;

1>	if all segments of the message have been received:

2>	assemble the message from the received segments and process the message according to 5.3.5.3 for the RRCReconfiguration message or 5.3.13.4 for the RRCResume message;

2>	discard all segments.

8.1.5.9.2.3	Test description

8.1.5.9.2.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

UE:

-	None

Preamble:

-	If pc_IP_Ping is set to TRUE then the UE is in 5GS state 1N-A, PDU SESSION ACTIVE according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-1

-	Else, the UE is in 5GS state 1N-A and Test Loop Function (On) with UE test loop mode B on NR Cell 1 according to 38.508-1[4], clause 4.4A.2 Table 4.4A.2-3.

8.1.5.9.2.3.2	Test procedure sequence

Table 8.1.5.9.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits a Paging message to the UE on the appropriate paging block, and including the UE identity in one entry of the IE pagingRecordList. | <-- | NR RRC: Paging | - | - |
| 2 | UE transmits an RRCSetupRequest message. | --> | NR RRC: RRCSetupRequest | - | - |
| 3 | SS transmits an RRCSetup message | <-- | NR RRC: RRCSetup | - | - |
| 4 | The UE transmits an RRCSetupComplete to confirm the successful completion of the connection establishment. | --> | NR RRC: RRCSetupComplete | - | - |
| 5 | SS transmits a SecurityModeCommand message to activate AS security. | <-- | NR RRC: SecurityModeCommand | - | - |
| 6 | Using the same slot as the SecurityModeCommand message in step 5, the SS transmits an DLDedicatedMessageSegment containing the first segment of the RRCReconfiguration message to establish a data radio bearer, DRB1 and signalling radio bearer, SRB2. | <-- | NR RRC: DLDedicatedMessageSegment | - | - |
| 7 | The UE transmits a SecurityModeComplete message. | --> | NR RRC: SecurityModeComplete | - | - |
| 8 | The SS transmits a DLDedicatedMessageSegment containing the second(last) segment of the RRCReconfiguration message. | <-- | NR RRC: DLDedicatedMessageSegment | - | - |
| 9 | Check: Does the UE transmit an RRCReconfigurationComplete message to confirm the establishment of data radio bearer, DRB1 and signalling radio bearer, SRB2? | --> | NR RRC: RRCReconfigurationComplete | 1 | P |
| 10 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB#1 established in Step 8? | - | - | 1 | - |
| 11 | The SS transmits a DLDedicatedMessageSegment containing the first segment of the RRCReconfiguration message to modify SRB and DRB configuration. | <-- | NR RRC: DLDedicatedMessageSegment |  |  |
| 12 | The SS transmits the second segment of the RRCReconfiguration message to modify SRB and DRB configuration. | <-- | NR RRC: DLDedicatedMessageSegment | - | - |
| 13 | The SS releases the RRC connection. | - | - | - | - |
| 14 | Step 1-8 of generic test procedure in TS 38.508-1 [4] subclause 4.5.4 to bring the UE in RRC_CONNECTED mode. | - | - | - | - |
| 15 | The SS transmits the third(last) segment of the RRCReconfiguration message to modify SRB2 and DRB configuration. | <-- | NR RRC: DLDedicatedMessageSegment | - | - |
| 16 | Check: Does the UE transmit an RRCReconfigurationComplete message in the next 3s? | --> | NR RRC: RRCReconfigurationComplete | 2 | F |
| 17 | The SS releases the RRC connection. | - | - | - | - |
| 18 | Step 1-8 of generic test procedure in TS 38.508-1 [4] subclause 4.5.4 to bring the UE in RRC_CONNECTED mode. | - | - | - | - |
| 19 | The SS transmits a DLDedicatedMessageSegment containing the first segment of the RRCReconfiguration message to modify SRB and DRB configuration. |  | NR RRC: DLDedicatedMessageSegment |  |  |
| 20 | The SS transmits the second segment of the RRCReconfiguration message to modify SRB and DRB configuration. | <-- | NR RRC: DLDedicatedMessageSegment | - | - |
| 21 | The SS transmits the third(last) segment of the RRCReconfiguration message to modify SRB2 and DRB configuration. | <-- | NR RRC: DLDedicatedMessageSegment | - | - |
| 22 | Check: Does the UE transmit an RRCReconfigurationComplete message in the next 3s? | --> | NR RRC: RRCReconfigurationComplete | 1 | P |
| 23 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB#1 established in Steps 12-21? | - | - | 1 |  |
| 24-27 | Void |  |  |  |  |
| NOTE 1: The request to establish a PDU session may be performed by MMI or AT Command. |  |  |  |  |  |

8.1.5.9.2.3.3	Specific message contents

Table 8.1.5.9.2.3.3-1: DLDedicatedMessageSegment (Step 6, 8, 11, 12, 15, 19, 20 and 21, Table 8.1.5.9.2.3.2-1)

| Derivation Path: TS 38.508-1[4], table 4.6.1-2B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DLDedicatedMessageSegment-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| dlDedicatedMessageSegment-r16 SEQUENCE { |  |  |  |
| segmentNumber-r16 | 0 |  | Step 6, Step 11, Step 19 |
| segmentNumber-r16 | 1 |  | Step 8, Step 12, Step 20 |
| segmentNumber-r16 | 2 |  | Step 15, Step 21 |
| rrc-MessageSegmentContainer-r16 | OCTET STRING (CONTAINING First segment of the RRCReconfiguration message) | Table 8.1.5.9.2.3.3-2 | Step 6 |
| rrc-MessageSegmentContainer-r16 | OCTET STRING (CONTAINING Last segment of the RRCReconfiguration message) | Table 8.1.5.9.2.3.3-2 | Step 8 |
| rrc-MessageSegmentContainer-r16 | OCTET STRING (CONTAINING First segment of the RRCReconfiguration message) | Table 8.1.5.9.2.3.3-4 | Step 11, Step 19 |
| rrc-MessageSegmentContainer-r16 | OCTET STRING (CONTAINING Second segment of the RRCReconfiguration message) | Table 8.1.5.9.2.3.3-4 | Step 12, Step 20 |
| rrc-MessageSegmentContainer-r16 | OCTET STRING (CONTAINING Last (third) segment of the RRCReconfiguration message) | Table 8.1.5.9.2.3.3-4 | Step 15, Step 21 |
| rrc-MessageSegmentType-r16 | notLastSegment |  | Step 6, Step 11, Step 19, Step 12, Step 20 |
| rrc-MessageSegmentType-r16 | LastSegment |  | Step 8, Step 15, Step 21 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.9.2.3.3-2: RRCReconfiguration (Step 6 and 8, Table 8.1.5.9.2.3.3-1)

| Derivation Path: TS 38.508-1[4], table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig with conditions SRB2 and DRB1 |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig Table 8.1.5.9.2.3.3-3) |  |
| lateNonCriticalExtension | '00,,,00'O | Add enough padding so that: maximum PDCP SDU size < encoded DLDedicatedMessageSegment size < 2 x maximum PDCP SDU size |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.9.2.3.3-3: CellGroupConfig (Table 8.1.5.9.2.3.3-2)

| Derivation Path: TS 38.508-1[4], table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-BearerConfig { | 2 entries |  |  |
| RLC-Bearer-Config[1] SEQUENCE { |  | entry 1 |  |
| logicalChannelIdentity | LogicalChannelIdentity |  |  |
| servedRadioBearer CHOICE { |  |  |  |
| drb-Identity | DRB-Identity with condition DRB1 |  |  |
| } |  |  |  |
| reestablishRLC | Not present |  |  |
| RLC-Config | RLC-Config with condition AM according to 38.508-1 [4], table 4.6.3-149 |  |  |
| mac-LogicalChannelConfig | LogicalChannelConfig according to 38.508-1 [4], table 4.6.3-66 |  |  |
| } |  |  |  |
| RLC-Bearer-Config[2] SEQUENCE { |  | entry 2 |  |
| logicalChannelIdentity | LogicalChannelIdentity |  |  |
| servedRadioBearer CHOICE { |  |  |  |
| srb-Identity | SRB-Identity with condition SRB2 |  |  |
| } |  |  |  |
| reestablishRLC | Not present |  |  |
| RLC-Config | Not present |  |  |
| mac-LogicalChannelConfig | LogicalChannelConfig according to 38.508-1 [4], table 4.6.3-66 |  |  |
| } |  |  |  |
| } |  |  |  |
| mac-CellGroupConfig | Not present |  |  |
| physicalCellGroupConfig | Not present |  |  |
| spCellConfig SEQUENCE | Not present |  |  |
| } |  |  |  |

Table 8.1.5.9.2.3.3-4: RRCReconfiguration (Step 11, 12, 15, 19, 20 and 21, Table 8.1.5.9.2.3.2-1)

| Derivation Path: TS 38.508-1[4], table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig Table 8.1.5.9.2.3.3-5) |  |
| } |  |  |  |
| lateNonCriticalExtension | '00,,,00'O | Add enough padding so that the size of the encoded DLDedicatedMessageSegment = 3 x maximum PDCP SDU size |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.9.2.3.3-5: CellGroupConfig (Table 8.1.5.9.2.3.3-3)

| Derivation Path: TS 38.508-1, table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-BearerConfig { | 2 entries |  |  |
| RLC-Bearer-Config[1] SEQUENCE { |  | entry 1 |  |
| logicalChannelIdentity | LogicalChannelIdentity |  |  |
| servedRadioBearer CHOICE { |  |  |  |
| srb-Identity | SRB-Identity with condition SRB2 |  |  |
| } |  |  |  |
| reestablishRLC | true |  |  |
| RLC-Config | RLC-Config with condition AM according to 38.508-1 [4], table 4.6.3-149 |  |  |
| mac-LogicalChannelConfig | LogicalChannelConfig according to 38.508-1 [4], table 4.6.3-66 |  |  |
| } |  |  |  |
| RLC-Bearer-Config[2] SEQUENCE { |  | entry 2 |  |
| logicalChannelIdentity | LogicalChannelIdentity |  |  |
| servedRadioBearer CHOICE { |  |  |  |
| drb-Identity | DRB-Identity with condition DRB1 |  |  |
| } |  |  |  |
| reestablishRLC | true |  |  |
| RLC-Config | Not present |  |  |
| mac-LogicalChannelConfig | LogicalChannelConfig according to 38.508-1 [4], table 4.6.3-66 |  |  |
| } |  |  |  |
| } |  |  |  |
| mac-CellGroupConfig | Not present |  |  |
| physicalCellGroupConfig | Not present |  |  |
| spCellConfig SEQUENCE | Not present |  |  |
| } |  |  |  |

##### 8.1.5.9.3	RRC resume / DL segment transfer

8.1.5.9.3.1	Test Purpose (TP)

(1)

with { UE in NR RRC_INACTIVE state, having sent an RRCResumeRequest message with resumeIdentity set to the stored shortI-RNTI value }

ensure that {

when { SS initiates the DL Dedicated Message Segment transfer procedure to send an RRCResume message }

then { UE successfully re-establishes the DRB and SRB}

}

(2)

with { UE in NR RRC_INACTIVE state, having sent an RRCResumeRequest message with resumeIdentity set to the stored fullI-RNTI value }

ensure that {

when { SS initiates the DL Dedicated Message Segment transfer procedure to send an RRCResume message }

then { UE successfully re-establishes the DRB and SRB}

}

8.1.5.9.3.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 38.331 clause 5.7.6.3. Unless otherwise stated these are Rel-15 requirements.

[TS 38.331, clause 5.7.6.3]

Upon receiving DLDedicatedMessageSegment message, the UE shall:

1>	store the segment included in rrc-MessageSegmentContainer;

1>	if all segments of the message have been received:

2>	assemble the message from the received segments and process the message according to 5.3.5.3 for the RRCReconfiguration message or 5.3.13.4 for the RRCResume message;

2>	discard all segments.

8.1.5.9.3.3	Test Description

8.1.5.9.3.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used.

UE:

-	None.

Preamble:

-	If pc_IP_Ping is set to TRUE then, the UE is in 5GS state 2N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

-	Else, the UE is in 5GS state 2N-A and Test Loop Function (On) with UE test loop mode B on NR Cell 1 according to 38.508-1[4], clause 4.4A.2 Table 4.4A.2-3.

8.1.5.9.3.3.2	Test procedure sequence

Table 8.1.5.9.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a Paging message including a matched identity (correct fullI-RNTI). | <-- | NR RRC: Paging | - | - |
| 2 | The UE transmit an RRCResumeRequest message by setting resumeIdentity to the stored shortI-RNTI value. | --> | NR RRC: RRCResumeRequest | - | - |
| 3 | The SS transmits a DLDedicatedMessageSegment containing the first segment of the 3 segment RRCResume message. | <-- | NR RRC: DLDedicatedMessageSegment | - | - |
| 4 | The SS transmits a DLDedicatedMessageSegment containing the second segment of the 3 segment RRCResume message. | <-- | NR RRC: DLDedicatedMessageSegment | - | - |
| 5 | The SS transmits a DLDedicatedMessageSegment containing the third (last) segment of the 3 segment RRCResume message. | <-- | NR RRC: DLDedicatedMessageSegment | - | - |
| 6 | The UE transmits an RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 7 | Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB#n associated with the first PDU session? | - | - | 1 | - |
| 8 | The SS transmits an RRCRelease message including both fullI-RNTI and shortI-RNTI in suspendConfig. | <-- | NR RRC: RRCRelease | - | - |
| 9 | The SS changes the SIB1 of NR Cell 1 to set the useFullResumeID to True. | - | - | - | - |
| 10 | The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. | <-- | PDCCH (DCI 1_0): Short Message | - | - |
| 11 | Wait for 2.1* modification period second for the UE to receive new system information. (Note 1) | - | - | - | - |
| 12 | The SS transmits a Paging message including a matched identity (correct fullI-RNTI). | <-- | NR RRC: Paging | - | - |
| 12 | The UE transmit an RRCResumeRequest1 message by setting resumeIdentity to the stored fullI-RNTI value? | --> | NR RRC: RRCResumeRequest1 | - | - |
| 13 | The SS transmits a DLDedicatedMessageSegment containing the first segment of the 2 segment RRCResume message. | <-- | NR RRC: DLDedicatedMessageSegment | - | - |
| 14 | The SS transmits a DLDedicatedMessageSegment containing the second/last segment of the 2 segment RRCResume message. | <-- | NR RRC: DLDedicatedMessageSegment |  |  |
| 15 | The UE transmits an RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 16 | Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB#n associated with the first PDU session? |  |  | 2 |  |
| 17 | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| Note 1: The modification period, expressed in number of radio frames = modificationPeriodCoeff * defaultPagingCycle. |  |  |  |  |  |

8.1.5.9.3.3.3	Specific message contents

Table 8.1.5.9.3.3.3-1: Paging (step 1, Table 8.1.5.9.3.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-9 with condition NR_RRC_RESUME

Table 8.1.5.9.3.3.3-2: DLDedicatedMessageSegment (Step 3, 4, 5, 13 and 14, Table 8.1.5.9.3.3.2-1)

| Derivation Path: TS 38.508-1[4], table 4.6.1-2B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DLDedicatedMessageSegment-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| dlDedicatedMessageSegment-r16 SEQUENCE { |  |  |  |
| segmentNumber-r16 | 0 |  | Step 3, Step 13 |
| segmentNumber-r16 | 1 |  | Step 4, Step 14 |
| segmentNumber-r16 | 2 |  | Step 5 |
| rrc-MessageSegmentContainer-r16 | OCTET STRING (CONTAINING First segment of the RRCResume message) | Table 8.1.5.9.3.3.3-3 | Step 3, Step 13 |
| rrc-MessageSegmentContainer-r16 | OCTET STRING (CONTAINING Second segment of the RRCResume message) | Table 8.1.5.9.3.3.3-3 | Step 4, Step 14 |
| rrc-MessageSegmentContainer-r16 | OCTET STRING (CONTAINING Third/Last segment of the RRCResume message) | Table 8.1.5.9.3.3.3-3 | Step 5 |
| rrc-MessageSegmentType-r16 | notLastSegment |  | Step 3, Step 4, Step 13 |
| rrc-MessageSegmentType-r16 | LastSegment |  | Step 5, Step 14 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.9.3.3.3-3: RRCResume (Table 8.1.5.9.3.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-17 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCResume ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcResume SEQUENCE { |  |  |  |
| lateNonCriticalExtension | '00,,,00'O | Add enough padding so that: 2 x maximum PDCP SDU size < encoded DLDedicatedMessageSegment size < 3 x maximum PDCP SDU size | Step 3, Step 4, Step 5 |
| lateNonCriticalExtension | '00,,,00'O | Add enough padding so that: maximum PDCP SDU size < encoded DLDedicatedMessageSegment size < 2 x maximum PDCP SDU size | Step 13, Step 14 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.9.3.3.3-4: RRCRelease (step 8, Table 8.1.5.9.3.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-16 with condition NR_RRC_INACTIVE

Table 8.1.5.9.3.3.3-5: SIB1 (step 9, Table 8.1.5.9.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| useFullResumeID | true |  |  |
| } |  |  |  |

Table 8.1.5.9.3.3.3-6: Paging (step 12, Table 8.1.5.9.3.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-9 with condition NR_RRC_RESUME

#### 8.1.5.10	UE Assistance Information

##### 8.1.5.10.1	UE Assistance Information/ Release Preference

8.1.5.10.1.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE configured to provide its release preference and a Preferred RRC State is provided (UE determines that it would prefer to transition out of RRC_CONNECTED) and timer T346f is not running }

then { the UE transmits a UEAssistanceInformation message }

}

(2)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE configured to provide its release preference and a Preferred RRC State is provided (UE determines that it would prefer to transition out of RRC_CONNECTED) and timer T346f is running }

then { the UE does not transmit a UEAssistanceInformation message }

}

(3)

with { UE in NR RRC_CONNECTED state and is configured with connectedReporting and having transmitted a UEAssistanceInformation message to indicate preference to transition out of RRC_CONNECTED state }

ensure that {

when { UE determines that it would prefer to remain in RRC_CONNECTED state and timer T346f is not  running }

then { the UE transmits a UEAssistanceInformation message }

}

8.1.5.10.1.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 38.331 clause 5.3.5.9, 5.7.4.2 and 5.7.4.3. Unless state explicitly these are Rel-16 requirements.

[TS 38.331, clause 5.3.5.9]

The UE shall:

…

1>	if the received otherConfig includes the releasePreferenceConfig:

2>	if releasePreferenceConfig is set to setup:

3>	consider itself to be configured to provide assistance information to transition out of RRC_CONNECTED in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide assistance information to transition out of RRC_CONNECTED and stop timer T346f, if running.

[TS 38.331, clause 5.7.4.2]

A UE capable of providing assistance information to transition out of RRC_CONNECTED state may initiate the procedure if it was configured to do so, upon determining that it prefers to transition out of RRC_CONNECTED state, or upon change of its preferred RRC state.

…

Upon initiating the procedure, the UE shall:

1>	if configured to provide its release preference and timer T346f is not running:

2>	if the UE determines that it would prefer to transition out of RRC_CONNECTED state; or

2>	if the UE is configured with connectedReporting and the UE determines that it would prefer to revert an earlier indication to transition out of RRC_CONNECTED state:

3>	start timer T346f with the timer value set to the releasePreferenceProhibitTimer;

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the release preference;

[TS 38.331, clause 5.7.4.3]

The UE shall set the contents of the UEAssistanceInformation message as follows:

…

1>	if transmission of the UEAssistanceInformation message is initiated to provide a release preference according to 5.7.4.2 or 5.3.5.3:

2>	include releasePreference in the UEAssistanceInformation message;

2>	set preferredRRC-State to the desired RRC state on transmission of the UEAssistanceInformation message;

8.1.5.10.1.3	Test Description

8.1.5.10.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used for NR cell 1.

UE:

-	None.

Preamble:

-	The UE is in 5GS state 3N-A on NR Cell 1 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

8.1.5.10.1.3.2	Test procedure sequence

Table 8.1.5.10.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message to configure releasePreferenceConfig with releasePreferenceProhibitTimer IE. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | The SS transmits an SET UAI REQUEST message indicating Preferred RRC State = ‘idle’. | <-- | SET UAI REQUEST | - | - |
| - | EXCEPTION: Step 4 and 5 can be performed in any order. | - | - | - | - |
| 4 | The UE transmits an SET UAI RESPONSE message. | --> | SET UAI RESPONSE | - | - |
| 5 | Check: Does the UE transmit an UEAssistanceInformation message to indicate IE preferredRRC-State-r16 = ‘idle’ within the ReleasePreference-r16 IE? | --> | NR RRC: UEAssistanceInformation | 1 | P |
| 6 | SS starts timer T346f = 30s. | - | - | - | - |
| 7 | The SS transmits an SET UAI REQUEST message indicating Preferred RRC State = ‘inactive’. | <-- | SET UAI REQUEST | - | - |
| 8 | The UE transmits an SET UAI RESPONSE message. | --> | SET UAI RESPONSE | - | - |
| 9 | Check: Does the UE transmit an UEAssistanceInformation message to indicate IE preferredRRC-State-r16 = ‘inactive’ within the ReleasePreference-r16 IE? | --> | NR RRC: UEAssistanceInformation | 2 | F |
| 10 | SS waits for timer T346f expiry. | - | - | - | - |
| - | EXCEPTION: Step 10a may be performed if the UE cached the preferredRRC-State indication from upper layers. |  |  |  |  |
| 10a | The UE transmit an UEAssistanceInformation message to indicate IE preferredRRC-State-r16 = ‘inactive’ within the ReleasePreference-r16 IE. | --> | NR RRC: UEAssistanceInformation | - | - |
| 11 | The SS transmits an SET UAI REQUEST message indicating Preferred RRC State = connected. | <-- | SET UAI REQUEST | - | - |
| 12 | The UE transmits an SET UAI RESPONSE message. | --> | SET UAI RESPONSE | - | - |
| 13 | Check: Does the UE transmit an UEAssistanceInformation message to indicate IE preferredRRC-State-r16 = ‘connected’ within the ReleasePreference-r16 IE? | --> | NR RRC: UEAssistanceInformation | 3 | F |
| 14 | The SS transmits an RRCReconfiguration message to configure releasePreferenceConfig with connectedReporting. | <-- | NR RRC: RRCReconfiguration | - | - |
| 15 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 16 | The SS transmits an SET UAI REQUEST message indicating Preferred RRC State = ‘connected’. | <-- | SET UAI REQUEST | - | - |
| - | EXCEPTION: Step 18 and 19 can be performed in any order. | - | - | - | - |
| 17 | The UE transmits an SET UAI RESPONSE message. | --> | SET UAI RESPONSE | - | - |
| 18 | Check: Does the UE transmit an UEAssistanceInformation message to indicate IE preferredRRC-State-r16 = ‘connected’ within the ReleasePreference-r16? | --> | NR RRC: UEAssistanceInformation | 3 | P |

8.1.5.10.1.3.3	Specific message contents

Table 8.1.5.10.1.3.3-1: RRCReconfiguration (step 1, Table 8.1.5.10.1.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| otherConfig-v1610 SEQUENCE { |  |  |  |
| releasePreferenceConfig-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| releasePreferenceProhibitTimer-r16 | s30 |  |  |
| connectedReporting | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.1.3.3-2: UEAssistanceInformation (Step 5, step 10a and step 18, Table 8.1.5.10.1.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-30 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEAssistanceInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueAssistanceInformation SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| releasePreference-r16 SEQUENCE { |  |  |  |
| preferredRRC-State-r16 | idle |  | Step 5 |
|  | inactive |  | Step 10a |
|  | connected |  | Step 18 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.1.3.3-3: RRCReconfiguration (step 14, Table 8.1.5.10.1.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| otherConfig-v1610 SEQUENCE { |  |  |  |
| releasePreferenceConfig-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| releasePreferenceProhibitTimer-r16 | s30 |  |  |
| connectedReporting | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.1.5.10.2	UE Assistance Information / MUSIM

8.1.5.10.2.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state}

ensure that {

when { UE configured to provide MUSIM assistance information for leaving RRC_CONNECTED and timer T346g is not running }

then { The UE transmits a UEAssistanceInformation message }

}

(2)

with { UE in NR RRC_CONNECTED state}

ensure that {

when { UE configured to provide MUSIM assistance information for leaving RRC_CONNECTED and timer T346g is running }

then { the UE Dose NOT transmit a UEAssistanceInformation message }

}

(3)

with { UE in NR RRC_CONNECTED state}

ensure that {

when { UE configured to provide MUSIM assistance information without leaving RRC_CONNECTED and the current musim-GapPreferenceList is different from the one indicated in the last transmission of the UEAssistanceInformation message including musim-GapPreferenceList }

then { The UE transmits a UEAssistanceInformation message }

}

8.1.5.10.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clause 5.7.4.1, 5.7.4.2, 5.7.4.3.

[TS 38.331, clause 5.3.5.9]

The UE shall:

…

1>	if the received otherConfig includes the musim-GapAssistanceConfig:

2>	if musim-GapAssistanceConfig is set to setup:

3>	consider itself to be configured to provide MUSIM assistance information for gap preference in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide MUSIM assistance information for gap preference and stop timer T346h, if running;

1>	if the received otherConfig includes the musim-LeaveAssistanceConfig:

2>	if musim-LeaveAssistanceConfig is set to setup:

3>	consider itself to be configured to provide MUSIM assistance information for leaving RRC_CONNECTED in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide MUSIM assistance information for leaving RRC_CONNECTED and stop timer T346g, if running.

[TS 38.331, clause 5.7.4.1]

Figure 5.7.4.1-1: UE Assistance Information

The purpose of this procedure is for the UE to inform the network of:

…

-	its preference to transition out of RRC_CONNECTED state for MUSIM operation, or;

-	its preference on the MUSIM gaps, or;

[TS 38.331, clause 5.7.4.2]

…

A UE capable of providing MUSIM assistance information for gap preference may initiate the procedure if it was configured to do so, upon determining it needs the gaps, or upon change of the gap preference information.

A UE capable of providing MUSIM assistance information for leave indication may initiate the procedure if it was configured to do so upon determining that it needs to leave RRC_CONNECTED state.

…

1>	if configured to provide MUSIM assistance information for leaving RRC_CONNECTED:

2>	if the UE needs to leave RRC_CONNECTED state and the timer T346g is not running:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide MUSIM assistance information for leaving RRC_CONNECTED;

3>	start the timer T346g with the timer value set to the musim-LeaveWithoutResponseTimer;

1>	if configured to provide MUSIM assistance information for gap preference:

2>	if the UE has a preference on the MUSIM gap(s) and the UE did not transmit a UEAssistanceInformation message with musim-GapPreferenceList since it was configured to provide MUSIM assistance information for gap preference; or

2>	if the current musim-GapPreferenceList is different from the one indicated in the last transmission of the UEAssistanceInformation message including musim-GapPreferenceList and the timer T346h is not running:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current musim-GapPreferenceList;

3>	start or restart the timer T346h with the timer value set to the musim-GapProhibitTimer.

NOTE 3:	The UE does not need to initiate transmission of the UAI message if the difference between the current musim-GapPreferenceList and the last transmission of the UEAssistanceInformation message including musim-GapPreferenceList is only due to removal of an ended aperiodic gap.

[TS 38.331, clause 5.7.4.3]

…

1>	if transmission of the UEAssistanceInformation message is initiated to provide MUSIM assistance information according to 5.7.4.2 or 5.3.5.3:

2>	if the UE has a preference for MUSIM periodic gap(s):

3>	include musim-GapPreferenceList with an entry for each periodic gap the UE prefers to be configured;

4>	set musim-GapLength and musim-GapRepetitionAndOffset in the musim-GapInfo IE to the values of the length and the repetition/offset of the gap(s), respectively, the UE prefers to be configured with;

2>	if the UE has a preference for MUSIM aperiodic gap:

3>	include the field musim-GapPreferenceList, with one entry for the aperiodic gap the UE prefers to be configured;

4>	include musim-GapLength in the musim-GapInfo IE and set it to the values of the length of the gap the UE prefers to be configured with;

4>	optionally include musim-Starting-SFN-AndSubframe in the musim-GapInfo IE and set it to the starting SFN/subframe of the gap the UE prefers to be configured with;

2>	if the UE has no longer preference for the periodic/aperiodic gaps:

3>	do not include musim-GapPreferenceList in the musim-Assistance IE;

2>	if UE has a preference to leave RRC_CONNECTED state:

3>	set musim-PreferredRRC-State to the preferred RRC state.

8.1.5.10.2.3	Test description

8.1.5.10.2.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1(home PLMN1) and NR Cell 12 (home PLMN2) are configured according to table 4.4.2-3 in TS 38.508-1 [18] and belong to the different frequency.

-	NR Cell 1 (belongs to TAI-1, home PLMN1, MCC=001,MNC=01) is set to ''Serving cell''.

-	NR Cell 12 (belongs to TAI-3, home PLMN2, MCC=001,MNC=02) is set to '' Serving cell''.

UE:

-	The UE is a MUSIM UE;

-	The UE is equipped with two USIMs with configuration as defined in TS 38.508-1 [4] Tables 6.4.1-27 and 6.4.1-28.

Preamble:

-	The UE performs a successful registration for MUSIM according to Table 4.9.36.2.2-1 of TS 38.508-1 [4].

8.1.5.10.2.3.2	Test procedure sequence

Table 8.1.5.10.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
|  | The following messages are to be observed on NR Cell 1 unless explicitly stated otherwise |  |  |  |  |
| 1-8 | The UE performs the generic procedure by executing steps 1 to 8 to enter the NR RRC_CONNECTED state as described in TS 38.508-1 [4] table 4.5.4.2-3 | - | - | - | - |
| 9 | The SS transmits an RRCReconfiguration message to configure musim-LeaveAssistanceConfig-r17 with musim-LeaveWithoutResponseTimer-r17 IE | <-- | NR RRC: RRCReconfiguration | - | - |
| 10 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
|  | EXCEPTION: Steps 11a1 and 11a2 are executed if UE supports test function specified in TS 38.509 [9] (pc_Set_MUSIM_UAI_Info_NR specified in TS 38.508-2[5]) ELSE Step 11b1 is executed(Note 1). |  |  |  |  |
| 11a1 | The SS transmits an SET MUSIM UAI REQUEST message indicating musim-PreferredRRC-State-r17 = ‘idle | <-- | SET MUSIM UAI REQUEST | - | - |
| 11a2 | The UE transmits an SET MUSIM UAI RESPONSE message. | --> | SET MUSIM UAI RESPONSE | - | - |
| 11b1 | Make the UE send UE Assistance information |  |  |  |  |
| 12 | Check: Does the UE transmits an UEAssistanceInformation message to indicate IE musim-PreferredRRC-State-r17 = ‘idle’ within musim-LeaveAssistanceConfig-r17 IE? | --> | NR RRC: UEAssistanceInformation | 1 | P |
| 13 | SS starts timer T346g = 100ms. | - | - | - | - |
|  | EXCEPTION: Steps 14a1 and 14a2 are executed if UE supports test function specified in TS 38.509 [9] (pc_Set_MUSIM_UAI_Info_NR specified in TS 38.508-2[5]) ELSE Step 14b1 is executed(Note 1). |  |  |  |  |
| 14a1 | The SS transmits an SET MUSIM UAI REQUEST message indicating musim-PreferredRRC-State-r17 = ‘inactive’ | <-- | SET MUSIM UAI REQUEST | - | - |
| 14a2 | The UE transmits an SET MUSIM UAI RESPONSE message. | --> | SET MUSIM UAI RESPONSE | - | - |
| 14b1 | Make the UE send UE Assistance information |  |  |  |  |
| 15 | Check: Does the UE transmit an UEAssistanceInformation message to indicate musim-PreferredRRC-State-r17 = ‘inactive’ within musim-LeaveAssistanceConfig-r17 IE? | --> | NR RRC: UEAssistanceInformation | 2 | F |
| 16 | SS waits for timer T346g expiry. | - | - | - | - |
| 17-24 | The UE performs the generic procedure by executing steps 1 to 8 to enter the NR RRC_CONNECTED state as described in TS 38.508-1 [4] table 4.5.4.2-3 | - | - | - | - |
| 25 | The SS transmits an RRCReconfiguration message to configure musim-GapProhibitTimer-r17 with musim-GapAssistanceConfig-r17 IE. | <-- | NR RRC: RRCReconfiguration | - | - |
| 26 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
|  | EXCEPTION: Steps 27a1 and 27a2 are executed if UE supports test function specified in TS 38.509 [9] (pc_Set_MUSIM_UAI_Info_NR specified in TS 38.508-2[5]) ELSE Step 27b1 is executed(Note 1). |  |  |  |  |
| 27a1 | The SS transmits an SET MUSIM UAI REQUEST message indicating musim-GapLength-r17 = ‘ms6’. | <-- | SET MUSIM UAI REQUEST | - | - |
| 27a2 | The UE transmits an SET MUSIM UAI RESPONSE message. | --> | SET MUSIM UAI RESPONSE | - | - |
| 27b1 | Make the UE send UE Assistance information |  |  |  |  |
| 28 | The UE transmit an UEAssistanceInformation message to indicate musim-GapLength-r17 = ‘ms6’ within MUSIM-GapPreferenceList-r17. | --> | NR RRC: UEAssistanceInformation | - | - |
|  | EXCEPTION: Steps 29a1 and 29a2 are executed if UE supports test function specified in TS 38.509 [9] (pc_Set_MUSIM_UAI_Info_NR specified in TS 38.508-2[5]) ELSE Step 29b1 is executed(Note 1). |  |  |  |  |
| 29a1 | The SS transmits an SET MUSIM UAI REQUEST message indicating musim-GapLength-r17 = ‘ms10’ after 1s. | <-- | SET MUSIM UAI REQUEST | - | - |
| 29a2 | The UE transmits an SET MUSIM UAI RESPONSE message. | --> | SET MUSIM UAI RESPONSE | - | - |
| 29b1 | Make the UE send UE Assistance information |  |  |  |  |
| 30 | Check: Does the UE transmit an UEAssistanceInformation message to indicate musim-GapLength-r17 = ‘ms10’ within MUSIM-GapPreferenceList-r17 IE? | --> | NR RRC: UEAssistanceInformation | 3 | P |
| Note1: The test function is optional, it can be replaced by alternative solutions (For Ex- AT command or MMI) due to UE implementation specific behavior. |  |  |  |  |  |

8.1.5.10.2.3.3	Specific message contents

Table 8.1.5.10.2.3.3-1: RRCReconfiguration (step 9, Table 8.1.5.10.2.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| otherConfig-v1700 SEQUENCE { |  |  |  |
| musim-LeaveAssistanceConfig-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| musim-LeaveWithoutResponseTimer-r17 | ms40 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.2.3.3-2: RRCReconfiguration (step 25, Table 8.1.5.10.2.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| otherConfig-v1700 SEQUENCE { |  |  |  |
| musim-GapAssistanceConfig-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| musim-GapProhibitTimer-r17 | s1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.2.3.3-3: UEAssistanceInformation (Step 12, Step 28, Step 30, Table 8.1.5.10.2.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-30 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEAssistanceInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueAssistanceInformation SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| musim-Assistance-r17 SEQUENCE { |  |  |  |
| musim-PreferredRRC-State-r17 | idle |  | Step 12 |
| musim-GapPreferenceList-r17 SEQUENCE (SIZE (1..4)) OF MUSIM-GapInfo-r17{ |  |  |  |
| MUSIM-GapInfo-r17 [1] SEQUENCE { |  |  |  |
| musim-Starting-SFN-AndSubframe-r17 | Not Present |  |  |
| musim-GapLength-r17 | ms6 |  | Step 28 |
|  | ms10 |  | Step 30 |
| musim-GapRepetitionAndOffset-r17 CHOICE { |  |  |  |
| ms80-r17 | 0 |  | Step 28Step 30 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.1.5.10.3	UE Assistance Information / MUSIM / Leaving RRC_CONNECTED / T346g expires

8.1.5.10.3.1	Test Purpose (TP)

(1)

with {  UE has transmitted the UEAssistanceInformation message with ”musim-PreferredRRC-State“ in MUSIM-Assistance-r17 IE that it prefers to idle state }

ensure that {

when { UE did not receive a response from the network until t346g expires }

then { UE autonomously leaves RRC_CONNECTED state and enters RRC_IDLE with release cause 'other' }

}

8.1.5.10.3.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clauses 5.3.5.9, 5.7.4.2 and 5.7.4.3. Unless otherwise stated these are Rel-17 requirements.

[TS 38.331, clause 5.3.5.9]

…

1>	if the received otherConfig includes the musim-LeaveAssistanceConfig:

2>	if musim-LeaveAssistanceConfig is set to setup:

3>	consider itself  to be configured to provide MUSIM assistance information for leaving RRC_CONNECTED in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide MUSIM assistance information for leaving RRC_CONNECTED and stop timer T346g, if running.

…

[TS 38.331, clause 5.7.4.2]

…

1>	if configured to provide MUSIM assistance information for leaving RRC_CONNECTED:

2>	if the UE needs to leave RRC_CONNECTED state and the timer T346g is not running:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide MUSIM assistance information for leaving RRC_CONNECTED;

3>	start the timer T346g with the timer value set to the musim-LeaveWithoutResponseTimer;

…

[TS 38.331, clause 5.7.4.3]

The UE shall set the contents of the UEAssistanceInformation message as follow

…

1>	if transmission of the UEAssistanceInformation message is initiated to provide MUSIM assistance information according to 5.7.4.2 or 5.3.5.3:

2>	if the UE has a preference for MUSIM periodic gap(s):

3>	include musim-GapPreferenceList with an entry for each periodic gap the UE prefers to be configured;

4>	set musim-GapLength and musim-GapRepetitionAndOffset in the musim-GapInfo IE to the values of the length and the repetition/offset of the gap(s), respectively, the UE prefers to be configured with;

2>	if the UE has a preference for MUSIM aperiodic gap:

3>	include the field musim-GapPreferenceList, with one entry for the aperiodic gap the UE prefers to be configured;

4>	include musim-GapLength in the musim-GapInfo IE and set it to the values of the length of the gap the UE prefers to be configured with;

4>	optionally include musim-Starting-SFN-AndSubframe in the musim-GapInfo IE and set it to the starting SFN/subframe of the gap the UE prefers to be configured with;

2>	if the UE has no longer preference for the periodic/aperiodic gaps:

3>	do not include musim-GapPreferenceList in the musim-Assistance IE;

2>	if UE has a preference to leave RRC_CONNECTED state:

3>	set musim-PreferredRRC-State to the preferred RRC state.

…

8.1.5.10.3.3	Test description

8.1.5.10.3.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1(home PLMN1) and NR Cell 12 (home PLMN2) are configured according to table 4.4.2-3 in TS 38.508-1 [18] and belong to the different frequency.-	NR Cell 1 (belongs to TAI-1, home PLMN1, MCC=001, MNC=01) is set to ''Serving cell''.

-	NR Cell 12 (belongs to TAI-3, home PLMN2, MCC=001, MNC=02) is set to '' Serving cell''.

UE:

-	The UE is a MUSIM UE;

-	The UE is equipped with two USIMs with configuration as defined in TS 38.508-1 [4] tables 6.4.1-27 and 6.4.1-28.

Preamble:

-	The UE performs a successful registration for MUSIM according to Table 4.9.36.2.2-1 of TS 38.508-1 [4].

-	The UE is brought to state 3N-A, RRC_CONNECTED Connectivity (NR), in accordance with the procedure described in TS 38.508-1 [4], clause 4.5.4 need to be performed on NR Cell 12.

8.1.5.10.3.3.2	Test procedure sequence

Table 8.1.5.10.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
|  | The following messages are to be observed on NR Cell 12 unless explicitly stated otherwise |  |  |  |  |
| 1 | The SS transmits an RRCReconfiguration message to configure musim-LeaveAssistanceConfig with musim-LeaveWithoutResponseTimer-r17 IE. | --> | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message. | <-- | NR RRC: RRCReconfigurationComplete | - | - |
|  | EXCEPTION: Steps 3a1 and 3a2 are executed if UE supports test function specified in TS 38.509 [9] (pc_Set_MUSIM_UAI_Info_NR) ELSE Step 3b1 is executed. (Note 1) |  |  |  |  |
| 3a1 | The SS transmits an SET UAI REQUEST message indicating Preferred RRC State = ‘idle’. | <-- | SET MUSIM UAI REQUEST | - | - |
| 3a2 | The UE transmits an SET UAI RESPONSE message. | --> | SET MUSIM UAI RESPONSE | - | - |
| 3b1 | Make the UE send UE Assistance information | - | - | - | - |
| 4 | Void |  |  |  |  |
| 5 | The UE transmits an UEAssistanceInformation message to indicate IE musim-PreferredRRC-State = ‘idle’ within the MUSIM-Assistance-r17 IE. | --> | NR RRC: UEAssistanceInformation |  |  |
| 6 | SS waits for timer T346g expiry. | - | - | - | - |
| 7 | Check: Does the test result of generic test procedure step 1-8 in TS 38.508-1 [4] Table 4.9.4.2.2-1 indicate that the UE is in RRC_IDLE? | - | - | 1 | P |
| Note1: The test function is optional, it can be replaced by alternative solutions (For Ex- AT command or MMI) due to UE implementation specific behavior. |  |  |  |  |  |

8.1.5.10.3.3.3	Specific message contents

Table 8.1.5.10.3.3.3-1: RRCReconfiguration (step 1, Table 8.1.5.10.3.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| otherConfig-v1700 CHOICE { |  |  |  |
| musim-LeaveAssistanceConfig-r17 SEQUENCE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| musim-LeaveWithoutResponseTimer-r17 | ms100 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.3.3.3-2: UEAssistanceInformation (Step 5, Table 8.1.5.10.3.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-30 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEAssistanceInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueAssistanceInformation SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| musim-Assistance-r17 |  |  |  |
| musim-PreferredRRC-State-r17 | idle |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.1.5.10.4	UE Assistance Information / RRM measurement relaxation / RedCap

Note: This test case is only applicable to certain specific UE implementation of RRM Relaxation.

8.1.5.10.4.1	Test Purpose (TP)

(1)

with { UE supporting RedCap and in NR RRC_Connected state and configured RRM measurement relaxation Stationary criterion }

ensure that {

when { the criterion is fulfilled and the UE did not transmit a UEAssistanceInformation message with rrm-MeasRelaxationFulfilment as true since it was configured to provide indications of RRM measurement relaxation criterion fulfilment }

then { the UE initiates transmission of the UEAssistanceInformation message to indicate that the criterion for RRM measurement relaxation for connected mode is fulfilled }

}

(2)

with { UE supporting RedCap and in NR RRC_Connected state and configured RRM measurement relaxation Stationary criterion }

ensure that {

when { the criterion is not fulfilled and the last UEAssistanceInformation message indicated fulfilment of the criterion with rrm-MeasRelaxationFulfilment as true}

then { the UE initiates transmission of the UEAssistanceInformation message to indicate that the criterion for RRM measurement relaxation for connected mode is not fulfilled }

}

(3)

with { UE supporting RedCap and in NR RRC_Connected state and configured RRM measurement relaxation Stationary criterion }

ensure that {

when { the criterion is fulfilled and the last UEAssistanceInformation message indicated the criterion is not fulfilled with rrm-MeasRelaxationFulfilment as false}

then { the UE initiates transmission of the UEAssistanceInformation message to indicate that the criterion for RRM measurement relaxation for connected mode is fulfilled}

}

8.1.5.10.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.304 clause 5.2.4.9 and TS 38.331 clause 5.7.4

[TS 38.304 clause 5.2.4.9]

The relaxed measurement criterion for a stationary RedCap UE is fulfilled when:

-	(SrxlevRefStationary – Srxlev) < SSearchDeltaP-Stationary,

Where:

-	Srxlev = current Srxlev value of the serving cell (dB).

-	SrxlevRefStationary = reference Srxlev value of the serving cell (dB), set as follows:

-	After selecting or reselecting a new cell, or

-	If (Srxlev - SrxlevRefStationary) > 0, or

-	If the relaxed measurement criterion has not been met for TSearchDeltaP-Stationary:

-	The UE shall set the value of SrxlevRefStationary to the current Srxlev value of the serving cell.

…

The relaxed measurement criterion for a stationary RedCap UE not at cell edge is fulfilled when:

-	the relaxed measurement criterion in clause 5.2.4.9.3 is fulfilled for a period of TSearchDeltaP-Stationary,

-	Srxlev > SSearchThresholdP2, and,

-	Squal > SSearchThresholdQ2, if SSearchThresholdQ2 is configured.

Where:

-	Srxlev = current Srxlev value of the serving cell (dB).

-	Squal = current Squal value of the serving cell (dB).

[TS 38.331 clause 5.7.4]

…

A UE capable of providing an indication of fulfilment of the RRM measurement relaxation criterion in connected mode may initiate the procedure if it was configured to do so, upon change of its fulfilment status for RRM measurement relaxation criterion for connected mode.

…

Upon initiating the procedure, the UE shall:

…

1>	if configured to send indications of RRM measurement relaxation criterion fulfilment:

2>	if the criterion in 5.7.4.4 is met for a period of TSearchDeltaP-StationaryConnected:

3>	if the UE did not transmit a UEAssistanceInformation message with rrm-MeasRelaxationFulfilment as true since it was configured to provide indications of RRM measurement relaxation criterion fulfilment; or

3>	the last UEAssistanceInformation message indicated the criterion in 5.7.4.4 is not fulfilled with rrm-MeasRelaxationFulfilment as false:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to indicate that the criterion for RRM measurement relaxation for connected mode is fulfilled;

2>	else:

3>	if the last UEAssistanceInformation message indicated fulfilment of the criterion in 5.7.4.4 with rrm-MeasRelaxationFulfilment as true:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to indicate that the criterion for RRM measurement relaxation for connected mode is not fulfilled.

…

The UE shall set the contents of the UEAssistanceInformation message as follows:

…

1>	if transmission of the UEAssistanceInformation message is initiated to provide an indication about whether the criterion for RRM relaxation for connected mode is fulfilled or not fulfilled:

2>	if the criterion for RRM measurement relaxation for connected mode is fulfilled:

3>	set the rrm-MeasRelaxationFulfilment to true;

2>	else:

3>	set the rrm-MeasRelaxationFulfilment to false.

…

The relaxed measurement criterion for a stationary UE is met when:

-	(SS-RSRPRefStationaryConnected – SS-RSRP) < SSearchDeltaP-StationaryConnected,

Where:

-	SS-RSRP = current L3 RSRP measurement of the PCell based on SSB (dB).

-	SS-RSRPRefStationaryConnected = reference SS-RSRP value of the PCell cell (dB), set as follows:

-	At the end of RRC reconfiguration procedure as specified in 5.3.5.3, when rrm-MeasRelaxationReportingConfig is included in the RRCReconfiguration message; or

-	If (SS-RSRP – SS-RSRPRefStationaryConnected) > 0; or

-	If the relaxed measurement criterion has not been met for TSearchDeltaP-StationaryConnected:

-	The UE shall set the value of SS-RSRPRefStationaryConnected to the current SS-RSRP value of the serving cell.

8.1.5.10.4.3	Test description

8.1.5.10.4.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

UE:

-	UE supporting RedCap

-	UE supporting RRM measurement relaxation

-	UE supporting initiation of UE Assistance Information procedure immediately upon change of its fulfilment status for RRM measurement relaxation criterion for connected mode.

Preamble:

-	The UE is in state 3N-A as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1.

8.1.5.10.4.3.2	Test procedure sequence

Table 8.1.5.10.4.3.2-1-1/2 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1", “T2” and "T3", are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.5.10.4.3.2-1: Time instances of cell power level and parameter changes in FR1

|  | Parameter | Unit | NR Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -94 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -88 | Power level is such that measurement relaxation Stationary criterion is fulfilled |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -94 | Power level is such that measurement relaxation Stationary criterion is not fulfilled |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | -88 | Power level is such that measurement relaxation Stationary criterion is fulfilled |

Table 8.1.5.10.4.3.2-2: Time instances of cell power level and parameter changes in FR2

|  | Parameter | Unit | NR Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -91 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -82 | Power level is such that measurement relaxation Stationary criterion is fulfilled |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -91 | Power level is such that measurement relaxation Stationary criterion is not fulfilled |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | -82 | Power level is such that measurement relaxation Stationary criterion is fulfilled |

Table 8.1.5.10.4.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message to setup RRM measurement relaxation | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | The SS re-adjusts the cell-specific reference signal level according to row "T1". | - | - | - | - |
| 4 | The SS waits for 5s (for t-SearchDeltaP-Stationary-r17 to expire). | - | - | - | - |
| 5 | Check: Does the UE transmit an UEAssistanceInformation message containing rrm-MeasRelaxationFulfilment-r17 set to “true”? | --> | NR RRC: UEAssistanceInformation | 1 | P |
| 6 | The SS re-adjusts the cell-specific reference signal level according to row "T2". | - | - | - | - |
| 7 | Check: Does the UE transmit an UEAssistanceInformation message containing rrm-MeasRelaxationFulfilment-r17 set to “false”? | --> | NR RRC: UEAssistanceInformation | 2 | P |
| 8 | The SS re-adjusts the cell-specific reference signal level according to row "T3". | - | - | - | - |
| 9 | The SS waits for 5s (for t-SearchDeltaP-Stationary-r17 to expire). | - | - | - | - |
| 10 | Check: Does the UE transmit an UEAssistanceInformation message containing rrm-MeasRelaxationFulfilment-r17 set to “true”? | --> | NR RRC: UEAssistanceInformation | 3 | P |

8.1.5.10.4.3.3	Specific message contents

Table 8.1.5.10.4.3.3-1: RRCReconfiguration (Step 1, Table 8.1.5.10.4.3.2-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| otherConfig-v1700 SEQUENCE { |  |  |  |
| ul-GapFR2-PreferenceConfig-r17 | Not present |  |  |
| musim-GapAssistanceConfig-r17 | Not present |  |  |
| musim-LeaveAssistanceConfig-r17 | Not present |  |  |
| successHO-Config-r17 | Not present |  |  |
| maxBW-PreferenceConfigFR2-2-r17 | Not present |  |  |
| maxMIMO-LayerPreferenceConfigFR2-2-r17 | Not present |  |  |
| minSchedulingOffsetPreferenceConfigExt-r17 | Not present |  |  |
| rlm-RelaxationReportingConfig-r17 | Not present |  |  |
| bfd-RelaxationReportingConfig-r17 | Not present |  |  |
| scg-DeactivationPreferenceConfig-r17 | Not present |  |  |
| rrm-MeasRelaxationReportingConfig-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| s-SearchDeltaP-Stationary-r17 | dB3 |  |  |
| t-SearchDeltaP-Stationary-r17 | s5 |  |  |
| } |  |  |  |
| } |  |  |  |
| propDelayDiffReportConfig-r17 | Not present |  |  |
| } |  |  |  |
| sl-L2RelayUE-Config-r17 | Not present |  |  |
| sl-L2RemoteUE-Config-r17 | Not present |  |  |
| dedicatedPagingDelivery-r17 | Not present |  |  |
| needForGapNCSG-ConfigNR-r17 | Not present |  |  |
| needForGapNCSG-ConfigEUTRA-r17 | Not present |  |  |
| musim-GapConfig-r17 | Not present |  |  |
| ul-GapFR2-Config-r17 | Not present |  |  |
| scg-State-r17 | Not present |  |  |
| appLayerMeasConfig-r17 | Not present |  |  |
| ue-TxTEG-RequestUL-TDOA-Config-r17 | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.4.3.3-2: UEAssistanceInformation (Steps 5 and 10,Table 8.1.5.10.4.3.2-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-30 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEAssistanceInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueAssistanceInformation SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rrm-MeasRelaxationFulfilment-r17 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.4.3.3-3: UEAssistanceInformation (Step 7,Table 8.1.5.10.4.3.2-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-30 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEAssistanceInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueAssistanceInformation SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rrm-MeasRelaxationFulfilment-r17 | false |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.1.5.10.5	UE Assistance Information / MUSIM / musim-cellToAffectList

8.1.5.10.5.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE is configured to provide MUSIM assistance information for temporary capability restriction and it has temporary capability restriction on the current configuration with timer T348 is not running }

then { UE transmits a UEAssistanceInformation message including musim-CellToAffectList and starts timer T348 with the timer value set to the musim-WaitTimer }

}

(2)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE is configured to provide MUSIM assistance information for temporary capability restriction and it has temporary capability restriction on the current configuration with timer T348 is running }

then { UE Does NOT transmit a UEAssistanceInformation message including musim-CellToAffectList }

}

8.1.5.10.5.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clause 5.7.4.2, and TS 38.300, clause 20.4. Unless otherwise stated these are Rel-18 requirements.

[TS 38.331, clause 5.7.4.2]

…

A UE capable of providing MUSIM assistance information for temporary capability restriction may initiate the procedure if it was configured to do so, upon determining it has temporary capability restriction or upon determining the removal of the capability restriction.

…

Upon initiating the procedure, the UE shall:

…

1>	if configured to provide MUSIM assistance information for temporary capability restriction:

2>	if the UE has temporary capability restriction on the current configuration and timer T348 is not running:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current musim-Cell-SCG-ToRelease and/or musim-CellToAffectList;

3>	start the timer T348 with the timer value set to the musim-WaitTimer.

[TS 38.300, clause 20.4]

For MUSIM operation, a MUSIM device in RRC_CONNECTED state in Network A may indicate its preference on temporary UE capability restriction or removal of restriction with Network A when the MUSIM device needs transmission or reception in Network B (e.g., including start/stop connection to Network B). Network A is NR and Network B can either be E-UTRA or NR. The MUSIM device may request a temporary capability restriction only after the Network signals via RRC that this is allowed.

When configured to do so, a MUSIM device can indicate one or more of the following temporary capability restriction or removal of restriction to the Network A:

-	A MUSIM device can explicitly request SCell(s) or SCG to be released;

-	A MUSIM device can indicate its preference on temporary maximum MIMO layers and/or supported channel bandwidth for specific serving cells for both UL/DL;

-	A MUSIM device can indicate its preference on the temporary maximum number of CCs per UL/DL;

-	A MUSIM device can indicate its preference on the concerned band(s) or band combination(s) (e.g. forbidden and/or affected band(s) or band combination(s)) based on a band-filter list configured by network. For affected band(s) and band combination(s), this preference can include temporary maximum MIMO layers and/or supported channel bandwidth for both UL/DL;

-	A MUSIM device can indicate the measurement gap requirement changes.

When it is allowed by Network A in SIB1, a MUSIM device can indicate to the Network A that its capabilities are temporarily restricted in RRCSetupComplete/RRCResumeComplete message while the MUSIM device is already in RRC_CONNECTED state in Network B.

When a MUSIM device is in RRC_CONNECTED state in both Network A and Network B, if Network B is NR, it is up to UE implementation to select which network to request temporary UE capability restriction; if Network B is E-UTRA, the request for temporary UE capability restriction can only be performed on Network A.

…

8.1.5.10.5.3	Test description

8.1.5.10.5.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1(home PLMN1) and NR Cell 12 (home PLMN2) are configured according to table 4.4.2-3 in TS 38.508-1 [18] and belong to the different frequency.

-	NR Cell 1 (belongs to TAI-1, home PLMN1, MCC=001,MNC=01) is set to ''Serving cell''.

-	NR Cell 12 (belongs to TAI-3, home PLMN2, MCC=001,MNC=02) is set to ''Serving cell ''.

UE:

-	The UE is a Rel-18 MUSIM UE;

-	The UE is equipped with two USIMs with configuration as defined in TS 38.508-1 [4] Tables 6.4.1-27 and 6.4.1-28.

Preamble:

-	The UE is in 5GS state 3N-A on NR Cell 1 and on NR Cell 12 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

-	The SIB1 of NR Cell 12 and NR Cell 1 are defined in TS 38.508-1 [4], clause 4.6.1-28 with condition MUSIM_r18 for all steps.

8.1.5.10.5.3.2	Test procedure sequence

Table 8.1.5.10.5.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | The following messages are to be observed on NR Cell 12 unless explicitly stated otherwise. | - | - | - | - |
| 1-6 | Void | - | - | - | - |
| 7 | The SS transmits an RRCReconfiguration message to trigger UE transmitting UEAssistanceInformation message. | <-- | NR RRC: RRCReconfiguration | - | - |
| 8 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 9 | Check: Does UE transmit UEAssistanceInformation message? | --> | NR RRC: UEAssistanceInformation | 1 | P |
| 10 | The SS transmits an RRCReconfiguration message to trigger UE transmitting UEAssistanceInformation. | <-- | NR RRC: RRCReconfiguration | - | - |
| 11 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 12 | Check: Does UE transmit UEAssistanceInformation message in 1s? | --> | NR RRC: UEAssistanceInformation | 2 | F |

8.1.5.10.5.3.3	Specific message contents

Table 8.1.5.10.5.3.3-1: RRCConnectionReconfiguration (step 7 and 10, Table 8.1.5.10.5.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| otherConfig-v1800 SEQUENCE { |  |  |  |
| musim-CapabilityRestrictionConfig-r18 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| musim-WaitTimer-r18 | ms100 |  |  |
| musim-ProhibitTimer-r18 | s10 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.5.3.3-2: UEAssistanceInformation (step 9, Table 8.1.5.10.5.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-30 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEAssistanceInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueAssistanceInformation SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| musim-Assistance-v1800 SEQUENCE { |  |  |  |
| musim-CapRestriction-r18 SEQUENCE { |  |  |  |
| musim-CellToAffectList-r18 | Any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.1.5.10.6	UE Assistance Information / MUSIM / musim-AffectedBandsList / musim-AvoidedBandsList / musim-MaxCC

8.1.5.10.6.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE is configured to provide MUSIM assistance information for temporary capability restriction and it has temporary capability restriction on the combination(s) of bands comprising of band(s) included in musim-CandidateBandList and it did not transmit a UEAssistanceInformation message with musim-AffectedBandsList and/or musim-AvoidedBandsList and/or musim-MaxCC since it was configured to provide MUSIM assistance information for temporary capability restriction and the timer T346n is not running }

then { UE transmits a UEAssistanceInformation message including current musim-AffectedBandsList and/or musim-AvoidedBandsList and/or musim-MaxCC and start or restart the timer 346n with the timer value set to the musim-ProhibitTimer }

}

(2)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE is configured to provide MUSIM assistance information for temporary capability restriction and the current musim-AffectedBandsList and/or musim-AvoidedBandsList and/or musim-MaxCC is different from the one indicated in the last transmission of the UEAssistanceInformation message including musim-CapRestriction and the timer T346n is not running }

then { UE transmits a UEAssistanceInformation message including current musim-AffectedBandsList and/or musim-AvoidedBandsList and/or musim-MaxCC and start or restart the timer T346n with the timer value set to the musim-ProhibitTimer }

}

(3)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE is configured to provide MUSIM assistance information for temporary capability restriction and it has temporary capability restriction on the current configuration and timer T346n is running }

then { UE Does NOT transmit a UEAssistanceInformation message including musim-AffectedBandsList and/or musim-AvoidedBandsList and/or musim-MaxCC }

}

8.1.5.10.6.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clause 5.7.4.2, and TS 38.300, clause 20.4. Unless otherwise stated these are Rel-18 requirements.

[TS 38.331, clause 5.7.4.2]

…

A UE capable of providing MUSIM assistance information for temporary capability restriction may initiate the procedure if it was configured to do so, upon determining it has temporary capability restriction or upon determining the removal of the capability restriction.

…

Upon initiating the procedure, the UE shall:

…

1>	if configured to provide MUSIM assistance information for temporary capability restriction:

2>	if the UE has temporary capability restriction on the current configuration and timer T348 is not running:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current musim-Cell-SCG-ToRelease and/or musim-CellToAffectList;

3>	start the timer T348 with the timer value set to the musim-WaitTimer.

2>	if the UE has temporary capability restriction on the combination(s) of bands comprising of band(s) included in musim-CandidateBandList or if the UE has temporary capability restriction on the maximum CC number, and the UE did not transmit a UEAssistanceInformation message with musim-AffectedBandsList and/or musim-AvoidedBandsList and/or musim-MaxCC since it was configured to provide MUSIM assistance information for temporary capability restriction and timer T346n is not running; or

2>	if the current musim-AffectedBandsList and/or musim-AvoidedBandsList and/or musim-MaxCC is different from the one indicated in the last transmission of the UEAssistanceInformation message including musim-CapRestriction and timer T346n is not running:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide the current musim-AffectedBandsList and/or musim-AvoidedBandsList and/or musim-MaxCC;

3>	start the timer T346n with the timer value set to the musim-ProhibitTimer.

[TS 38.300, clause 20.4]

For MUSIM operation, a MUSIM device in RRC_CONNECTED state in Network A may indicate its preference on temporary UE capability restriction or removal of restriction with Network A when the MUSIM device needs transmission or reception in Network B (e.g., including start/stop connection to Network B). Network A is NR and Network B can either be E-UTRA or NR. The MUSIM device may request a temporary capability restriction only after the Network signals via RRC that this is allowed.

When configured to do so, a MUSIM device can indicate one or more of the following temporary capability restriction or removal of restriction to the Network A:

-	A MUSIM device can explicitly request SCell(s) or SCG to be released;

-	A MUSIM device can indicate its preference on temporary maximum MIMO layers and/or supported channel bandwidth for specific serving cells for both UL/DL;

-	A MUSIM device can indicate its preference on the temporary maximum number of CCs per UL/DL;

-	A MUSIM device can indicate its preference on the concerned band(s) or band combination(s) (e.g. forbidden and/or affected band(s) or band combination(s)) based on a band-filter list configured by network. For affected band(s) and band combination(s), this preference can include temporary maximum MIMO layers and/or supported channel bandwidth for both UL/DL;

-	A MUSIM device can indicate the measurement gap requirement changes.

When it is allowed by Network A in SIB1, a MUSIM device can indicate to the Network A that its capabilities are temporarily restricted in RRCSetupComplete/RRCResumeComplete message while the MUSIM device is already in RRC_CONNECTED state in Network B.

When a MUSIM device is in RRC_CONNECTED state in both Network A and Network B, if Network B is NR, it is up to UE implementation to select which network to request temporary UE capability restriction; if Network B is E-UTRA, the request for temporary UE capability restriction can only be performed on Network A.

…

8.1.5.10.6.3	Test description

8.1.5.10.6.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1(home PLMN1), NR Cell 10 (home PLMN1) and NR Cell 12 (home PLMN2) are configured according to table 4.4.2-3 in TS 38.508-1 [18] and belong to the different frequencies.

-	NR Cell 1 and NR Cell 10 belong to TAI-1, home PLMN1, MCC=001, MNC=01.

-	NR Cell 12 belongs to TAI-3, home PLMN2, MCC=001, MNC=02.

-	The SIB1 of NR Cell 12 and NR Cell 1 are defined in TS 38.508-1 [4], clause 4.6.1-28 with condition MUSIM_r18 for all steps.

-	System information combination NR-4 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 with NR Cell 10 replaces NR Cell 3 is used to NR cell1.

UE:

-	The UE is a Rel-18 MUSIM UE;

-	The UE is equipped with two USIMs with configuration as defined in TS 38.508-1 [4] Tables 6.4.1-27 and 6.4.1-28.

Preamble:

-	The UE is in 5GS state 3N-A on NR Cell 12 and on NR Cell 1 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

8.1.5.10.6.3.2	Test procedure sequence

Tables 8.1.5.10.6.3.2-1 illustrates the downlink power levels to be applied for NR Cell 1, NR Cell 1, and NR Cell 12 at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble.

Table 8.1.5.10.6.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 10 | NR Cell 12 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | “Off” | -88 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -88 | -88 |  |

Table 8.1.5.10.6.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | The following messages are to be observed on NR Cell 1 unless explicitly stated otherwise. | - | - | - | - |
| 1-6 | Void | - | - | - | - |
| 7 | The SS transmits an RRCReconfiguration message to trigger UE transmitting UEAssistanceInformation message. | <-- | NR RRC: RRCReconfiguration | - | - |
| 8 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 9 | Check: Does the UE transmit UEAssistanceInformation message? | --> | NR RRC: UEAssistanceInformation | 1 | P |
| 10 | The SS adjusts the cell power levels according to row "T1" in table 8.1.5.10.6.3.2-1. | - |  | - | - |
| 11 | The SS transmits an RRCReconfiguration message for handover to NR Cell 10 and trigger UE transmitting UEAssistanceInformation. | <-- | NR RRC: RRCReconfiguration | - | - |
| - | The following messages are to be observed on NR Cell 10 unless explicitly stated otherwise. | - | - | - | - |
| 12 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 13 | Check: Does UE transmit UEAssistanceInformation message? | --> | NR RRC: UEAssistanceInformation | 3 | F |
| 14 | Wait 5s to ensure T346n expire. |  |  |  |  |
| 15 | The SS transmits an RRCReconfiguration message to trigger UE transmitting UEAssistanceInformation message. | <-- | NR RRC: RRCReconfiguration | - | - |
| 16 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 17 | Check: Does UE transmit UEAssistanceInformation message? | --> | NR RRC: UEAssistanceInformation | 2 | P |

8.1.5.10.6.3.3	Specific message contents

Table 8.1.5.10.6.3.3-1: RRCConnectionReconfiguration (step 7 and step 15, Table 8.1.5.10.6.3.2-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| otherConfig-v1800 SEQUENCE { |  |  |  |
| musim-CapabilityRestrictionConfig-r18 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| musim-CandidateBandList-r18 (SIZE(1.. maxCandidateBandIndex-r18)) OF FreqBandIndicatorNR { | 2 entries |  |  |
|  | FreqBandIndicatorNR of the band of NR Cell1 |  | pc_nrBandx ('x' being the band number/type related PICS listed in TS 38.508-2 [5]) |
|  | FreqBandIndicatorNR of the band of NR Cell10 |  |  |
| } |  |  |  |
| musim-WaitTimer-r18 | ms100 |  |  |
| musim-ProhibitTimer-r18 | s5 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.6.3.3-2: UEAssistanceInformation (step 9 and step 17, Table 8.1.5.10.6.3.2-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-30 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEAssistanceInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueAssistanceInformation SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| musim-Assistance-v1800 SEQUENCE { |  |  |  |
| musim-CapRestriction-r18 SEQUENCE { |  |  |  |
| musim-AffectedBandsList-r18 | Not present or any allowed value |  |  |
| musim-AvoidedBandsList-r18 | Any allowed value |  | If musim-AffectedBandsList-r18 is not present |
| musim-AvoidedBandsList-r18 | Not present or any allowed value |  | If musim-AffectedBandsList-r18 is present |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.6.3.3-3: RRCConnectionReconfiguration (step 11, Table 8.1.5.10.6.3.2-2)

| Derivation Path: TS 38.508-1 [4] Table 4.8.1-1A with condition RBConfig_NoKeyChange |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| otherConfig-v1800 SEQUENCE { |  |  |  |
| musim-CapabilityRestrictionConfig-r18 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| musim-CandidateBandList-r18 (SIZE(1.. maxCandidateBandIndex-r18)) OF FreqBandIndicatorNR { | 2 entries |  |  |
|  | FreqBandIndicatorNR of the band of NR Cell1 |  | pc_nrBandx ('x' being the band number/type related PICS listed in TS 38.508-2 [5]) |
|  | FreqBandIndicatorNR of the band of NR Cell10 |  |  |
| } |  |  |  |
| musim-WaitTimer-r18 | ms100 |  |  |
| musim-ProhibitTimer-r18 | s5 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.1.5.10.7	UE Assistance Information / UL Traffic Information

8.1.5.10.7.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE configured to provide UL traffic information and the UL traffic information is available, and UE did not transmit a UEAssistanceInformation message with ul-TrafficInfo since it was configured to provide UL traffic information }

then { the UE transmits a UEAssistanceInformation message }

}

(2)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE configured to provide UL traffic information and the UL traffic information has changed with respect to the last reporting (UE determines that the information has been updated) and timer T346l is running }

then { the UE does not transmit a UEAssistanceInformation message }

}

(3)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE configured to provide UL traffic information and the UL traffic information has changed with respect to the last reporting (UE determines that the information has been updated) and timer T346l is not running }

then { the UE transmits a UEAssistanceInformation message }

}

8.1.5.10.7.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.331, clause 5.3.1. Unless otherwise stated these are Rel-18 requirements.

[TS 38.331, clause 5.7.4.1]

The purpose of this procedure is for the UE to inform the network of:

…

-	UL traffic information; or

…

[TS 38.331, clause 5.7.4.2]

A UE capable of providing UL traffic information shall initiate the procedure when this information is available upon being configured to do so, and upon change of UL traffic information.

…

Upon initiating the procedure, the UE shall:

…

1>	if configured to provide UL traffic information:

2>	if the UE did not transmit a UEAssistanceInformation message with ul-TrafficInfo since it was configured to provide UL traffic information; or

2>	if UL traffic information included in the previous UEAssistanceInformation has changed since the last transmission of the UEAssistanceInformation message containing ul-TrafficInfo for at least one QoS flow for which timer T346l is not running:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide UL traffic information.

NOTE 5:	The UE only considers burstArrivalTime to have changed when it changes relative to the periodicity of the Data Burst arrival.

[TS 38.331, clause 5.7.4.3]

The UE shall set the contents of the UEAssistanceInformation message as follows:

1>	if transmission of the UEAssistanceInformation message is initiated to provide UL traffic information according to 5.7.4.2 or 5.3.5.3:

2>	for each PDU session for which the UE intends to provide UL traffic information in this UEAssistanceInformation message:

3>	set pdu-SessionID to the value of the concerned PDU session ID;

3>	if transmission of the UEAssistanceInformation message is initiated to provide UL traffic information according to 5.3.5.3:

4>	stop timer T346l for each QoS flow of this PDU session for which the UE intends to provide UL traffic information in this UEAssistanceInformation message;

3>	for each QoS flow of this PDU session for which timer T346l is not running and for which the UE intends to provide UL traffic information in this UEAssistanceInformation message:

4>	start timer T346l associated to this QoS flow with the timer value set to the value of ul-TrafficInfoProhibitTimer;

4>	set qfi to the value of the concerned QFI;

4>	if the jitter range measurement is available; and

4>	if the UE did not provide jitter range since it was configured to provide UL traffic information, or if the measured jitter range has changed since the last transmission of the UEAssistanceInformation message containing jitterRange:

5>	set jitterRange to the latest measured value of the jitter range;

4>	if the burst arrival time measurement is available; and

4>	if the UE did not provide burst arrival time since it was configured to provide UL traffic information, or if the measured burst arrival time has changed since the last transmission of the UEAssistanceInformation message containing burstArrivalTime:

5>	set burstArrivalTime to the latest measured value of the burst arrival time;

4>	if the traffic periodicity measurement is available; and

4>	if the UE did not provide traffic periodicity since it was configured to provide UL traffic information, or if the measured traffic periodicity has changed since the last transmission of the UEAssistanceInformation message containing trafficPeriodicity:

5>	set trafficPeriodicity to the latest measured value of the traffic periodicity;

4>	if the UE did not provide pdu-SetIdentification since it was configured to provide UL traffic information, or if the information previously provided in pdu-SetIdentification has changed since the last transmission of the UEAssistanceInformation message containing pdu-SetIdentification:

5>	if the UE is able to identify PDU Set(s) for the QoS flow:

6>	set pdu-SetIdentification to true;

5>	else:

6>	set pdu-SetIdentification to false.

4>	if the UE did not provide psi-Identification since it was configured to provide UL traffic information, or if the information previously provided in psi-Identification has changed since the last transmission of the UEAssistanceInformation message containing psi-Identification:

5>	if the UE is able to identify PSI(s) for the QoS flow:

6>	set psi-Identification to true;

5>	else:

6>	set psi-Identification to false.

[TS 38.331, clause 5.3.5.9]

1>	if the received otherConfig includes the ul-TrafficInfoReportingConfig:

2>	if ul-TrafficInfoReportingConfig is set to setup:

3>	consider itself to be configured to provide UL traffic information in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide UL traffic information and stop all instances of timer T346l, if running;

[TS 38.331, clause 7.1.1]

| Timer | Start | Stop | At expiry |
| --- | --- | --- | --- |
| T346l (The UE maintains one instance of this timer per QoS flow) | Upon transmitting UEAssistanceInformation message with ul-TrafficInfo for the concerned QoS flow. | Upon releasing ul-TrafficInfoReportingConfig during the connection re-establishment/resume procedures, or upon receiving ul-TrafficInfoReportingConfig set to release. | No action. |

8.1.5.10.7.3	Test description

8.1.5.10.7.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cell 1.

UE:

-	None

Preamble:

-	The UE is in 5GS state 3N-A with one PDU session active on NR Cell 1 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

8.1.5.10.7.3.2	Test procedure sequence

Table 8.1.5.10.7.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message to configure ul-TrafficInfoReportingConfig. | --> | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message. | <-- | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | Set UL Traffic information for QFI 1. (Note 1) | - | - | - | - |
| 4 | Check: Does the UE transmit an UEAssistanceInformation message including UL-TrafficInfo-r18? | --> | NR RRC: UEAssistanceInformation | 1 | P |
| 5 | Change UL Traffic information for QFI 1. (Note 1) | - | - | - | - |
| 6 | Check: Does the UE transmit an UEAssistanceInformation message before T346l expiry? | - | - | 3 | F |
| 7 | SS waits for timer T346l expiry. | - | - | - | - |
| 8 | Check: Does the UE transmit an UEAssistanceInformation message including UL-TrafficInfo-r18? | --> | NR RRC: UEAssistanceInformation | 2 | P |
| Note 1: The request to set or change UL Traffic information is performed by MMI or AT Command. UL Traffic information includes at least one of jitter range, burst arrival time, data burst periodicity and PDU Set and PSI identification. |  |  |  |  |  |

8.1.5.10.7.3.3	Specific message contents

Table 8.1.5.10.7.3.3-1: RRCReconfiguration (step 1, Table 8.1.5.10.7.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| otherConfig-v1800 SEQUENCE { |  |  |  |
| ul-TrafficInfoReportingConfig-r18 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| pdu-SessionsToReportUL-TrafficInfoList-r18 SEQUENCE (SIZE (1.. maxNrofPDU-Sessions-r17)) OF PDU-SessionToReportUL-TrafficInfo-r18 { | 1 entry |  |  |
| PDU-SessionToReportUL-TrafficInfo-r18 [1] SEQUENCE { |  | entry 1 |  |
| pdu-SessionID | The same value as the value set in internet PDU session |  |  |
| qfi-ToReportUL-TrafficInfoList-r18 SEQUENCE (SIZE (1..maxNrofQFIs)) OF QFI { | 1 entry |  |  |
| QFI[1] | ’00 0001’B | entry 1 QFI 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ul-TrafficInfoProhibitTimer-r18 | s30 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.7.3.3-2: UEAssistanceInformation (step 4 and step 8, Table 8.1.5.10.7.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-30 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEAssistanceInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueAssistanceInformation SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ul-TrafficInfo-r18 SEQUENCE (SIZE (1..maxNrofPDU-Sessions-r17)) OF PDU-SessionUL-TrafficInfo-r18 { | 1 entry |  |  |
| PDU-SessionUL-TrafficInfo-r18[1] SEQUENCE { |  | entry 1 |  |
| pdu-SessionID-r18 | The same value as the value set in internet PDU session |  |  |
| qos-FlowUL-TrafficInfoList-r18 SEQUENCE (SIZE (1..maxNrofQFIs)) OF QOS-FlowUL-TrafficInfo-r18 { | 1 entry |  |  |
| qos-FlowUL-TrafficInfoList-r18[1] SEQUENCE { |  | entry 1 |  |
| qfi-r18 | ’00 0001’B | QFI 1 |  |
| jitterRange-r18 | Not checked |  |  |
| burstArrivalTime-r18 | Not checked |  |  |
| trafficPeriodicity-r18 | Not checked |  |  |
| pdu-SetIdentification-r18 | Present but contents not checked |  | Step 4 |
|  | Not checked |  | Step 8 |
| psi-Identification-r18 | Present but contents not checked |  | Step 4 |
|  | Not checked |  | Step 8 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| Note 1: ul-TrafficInfo-r18 in step8 is different from the ul-TrafficInfo-r18 in step4 because the UL Traffic information is changed in step 5. |  |  |  |

##### 8.1.5.10.8	UE Assistance Information / IDC mechanism

8.1.5.10.8.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE configured to provide IDC assistance information and the UE is experiencing IDC problems that it cannot solve by itself and UE did not transmit a UEAssistanceInformation message with idc-Assistance }

then { UE transmits a UEAssistanceInformation message to provide IDC assistance information including IDC-Assistance-r16 }

}

(2)

with { UE in NR RRC_CONNECTED state and UE already transmits a UEAssistanceInformation message with idc-Assistance }

ensure that {

when { UE configured to provide IDC assistance information and the UE is experiencing IDC problems that it cannot solve by itself and the current idc-Assistance information is different from the one indicated in the last UEAssistanceInformation message }

then { UE transmits a UEAssistanceInformation message to provide IDC assistance information including IDC-Assistance-r16 }

}

8.1.5.10.8.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in TS 38.331, clause 5.3.5.9, 5.7.4.2 and 5.4.7.3. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.3.5.9]

…

1>	if the received otherConfig includes the idc-AssistanceConfig:

2>	if idc-AssistanceConfig is set to setup:

3>	consider itself to be configured to provide IDC assistance information in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide IDC assistance information;

…

[TS 38.331, clause 5.7.4.2]

Upon initiating the procedure, the UE shall:

1>	if configured to provide delay budget report:

…

1>	if configured to provide IDC assistance information based on candidateServingFreqListNR included in idc-AssistanceConfig of a cell group:

2>	if the UE did not transmit a UEAssistanceInformation message with idc-Assistance since it was configured to provide IDC assistance information:

3>	if on one or more frequencies included in candidateServingFreqListNR, the UE is experiencing IDC problems that it cannot solve by itself; or

3>	if on one or more supported UL CA or NR-DC combination comprising of carrier frequencies included in candidateServingFreqListNR, the UE is experiencing IDC problems that it cannot solve by itself:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information;

2>	else if the current idc-Assistance information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information;

…

[TS 38.331, clause 5.7.4.3]

The UE shall set the contents of the UEAssistanceInformation message as follows:

1>	if transmission of the UEAssistanceInformation message is initiated to provide a delay budget report according to 5.7.4.2 or 5.3.5.3;

...

1>	if transmission of the UEAssistanceInformation message is initiated to provide IDC assistance information according to 5.7.4.2 or 5.3.5.3:

2>	if there is at least one carrier frequency included in candidateServingFreqListNR, the UE is experiencing IDC problems that it cannot solve by itself:

3>	include the field affectedCarrierFreqList with an entry for each affected carrier frequency included in candidateServingFreqListNR;

3>	for each carrier frequency included in the field affectedCarrierFreqList, include interferenceDirection and set it accordingly;

2>	if there is at least one supported UL CA or NR-DC combination comprising of carrier frequencies included in candidateServingFreqListNR, the UE is experiencing IDC problems that it cannot solve by itself:

3>	include victimSystemType for each UL CA or NR-DC combination included in affectedCarrierFreqCombList;

3>	if the UE sets victimSystemType to wlan or bluetooth:

4>	include affectedCarrierFreqCombList with an entry for each supported UL CA combination comprising of carrier frequencies included in candidateServingFreqListNR, that is affected by IDC problems;

3>	else:

4>	optionally include affectedCarrierFreqCombList with an entry for each supported UL CA or NR-DC combination comprising of carrier frequencies included in candidateServingFreqListNR, that is affected by IDC problems;

...

8.1.5.10.8.3	Test description

8.1.5.10.8.3.1	Pre-test conditions

System Simulator:

-	NR Cell 14/NRf4 (2390-2400MHz for band n40) and WLAN AP 1 (Cell 27) as specified in TS 38.508-1[4] clause 4.13.

-	NR Cell 1/NRf1 (2496-2556MHz for band n41) and WLAN AP 2 (Cell 27b) as specified in TS 38.508-1[4] clause 4.13.

UE:

-	UE’s WLAN is on.

Preamble:

-	The UE is in state 3N-A on NR Cell 14 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

8.1.5.10.8.3.2	Test procedure sequence

Table 8.1.5.10.8.3.2-1 for FR1 illustrate the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. The configuration "T0" indicates the initial conditions. Subsequent configurations marked "T1", "T2" and "T3" are applied at the points indicated in the Main behaviour description in Table 8.1.5.10.8.3.2-2.

Table 8.1.5.10.8.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 14 | WLAN AP 1 (Cell 27) | NR Cell 1 | WLAN AP 2 (Cell 27b) | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 |  | Off |  | Cell configuration for non-IDC. |
|  | BeaconRSSI | dBm |  | Off |  | Off |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -88 |  | Off |  | Cell configuration for IDC in NR Cell 14. |
|  | BeaconRSSI | dBm |  | -80 |  | Off |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -88 |  | -79 |  | Cell configuration for non-IDC in NR Cell 1. |
|  | BeaconRSSI | dBm |  | Off |  | Off |  |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | -88 |  | -79 |  | Cell configuration for IDC in NR Cell 1. |
|  | BeaconRSSI | dBm |  | Off |  | -80 |  |

Table 8.1.5.10.8.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a RRCReconfiguration message including OtherConfig with idc-AssistanceConfig-r16 including CandidateServingFreqListNR-r16 including center frequency for NR Cell 14 and NR Cell 1. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | The SS re-adjusts the power levels according to row "T1" in Table 8.1.5.10.8.3.2-1. | - | - | - | - |
| 4 | Void | - | - | - | - |
| 5 | The UE associates with the WLAN AP1 and obtains the local IP address of WLAN AP1. Note: The UL signal to WLAN AP shall generate IDC problem. The UE shall detect IDC problem with “T1” in RRC_CONNECTED in NR Cell 14. |  |  |  |  |
| 6 | Check: Does the UE transmit an an UEAssistanceInformation message including IDC-Assistance-r16 with at least one affected carrier frequency including the carrier frequency of NR Cell 14 in affectedCarrierFreqList? | --> | NR RRC: UEAssistanceInformation | 1 | P |
| 7 | Void | - | - | - | - |
| 8 | The SS re-adjusts the power levels according to row "T2" in Table 8.1.5.10.8.3.2-1. | - | - | - | - |
| 9 | Void | - | - | - | - |
| 10 | The SS transmits an RRCReconfiguration message including to order the UE to perform inter-frequency handover to NR Cell 1 | <-- | NR RRC: RRCReconfiguration | - | - |
| 11 | The UE transmit RRCReconfigurationComplete message in NR Cell 1. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 12 | The SS re-adjusts the power levels according to row "T3" in Table 8.1.5.10.8.3.2-1. | - | - | - | - |
| 13 | The UE associates with the WLAN AP2 and obtains the local IP address of WLAN AP2. Note: The UL signal to WLAN AP shall generate IDC problem. The UE shall detect IDC problem with “T3” in RRC_CONNECTED in NR Cell 1. |  |  |  |  |
| 14 | Check: Does the UE transmit an an UEAssistanceInformation message including IDC-Assistance-r16 with at least one affected carrier frequency including the carrier frequency of NR Cell 1 in affectedCarrierFreqList, which is different from the one in Step 6? | --> | NR RRC: UEAssistanceInformation | 2 | P |

8.1.5.10.8.3.3	Specific message contents

Table 8.1.5.10.8.3.3-1: RRCReconfiguration (step 1, Table 8.1.5.10.8.3.2-2)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| otherConfig-v1610 SEQUENCE { |  |  |  |
| idc-AssistanceConfig-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| candidateServingFreqListNR-r16 SEQUENCE (SIZE (1..maxFreqIDC-r16)) OF ARFCN-ValueNR { | 2 entries |  |  |
| ARFCN-ValueNR[1] | ARFCN-ValueNR of NR Cell 14 |  |  |
| ARFCN-ValueNR[2] | ARFCN-ValueNR of NR Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.8.3.3-2: Void

Table 8.1.5.10.8.3.3-3: Void

Table 8.1.5.10.8.3.3-4: UEAssistanceInformation (step 6, Table 8.1.5.10.8.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-30 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEAssistanceInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueAssistanceInformation SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| overheatingAssistance | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| idc-Assistance-r16 SEQUENCE { |  |  |  |
| affectedCarrierFreqList-r16 SEQUENCE { | 1 enrty |  |  |
| AffectedCarrierFreq-r16 SEQUENCE { |  |  |  |
| carrierFreq-r16 | ARFCN-ValueNR of NR Cell 14 |  |  |
| interferenceDirection-r16 | nr |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.8.3.3-5: Void

Table 8.1.5.10.8.3.3-6: RRCReconfiguration (step 10, Table 8.1.5.10.8.3.2-2)

| Derivation Path: TS 38.508-1 [4] 4.8.1-1A with condition RBConfig_KeyChange |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.8.3.3-7: CellGroupConfig (Table 8.1.5.10.8.3.3-6)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with condition PCell_change and CFRA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| spCellConfigCommon SEQUENCE { |  |  |  |
| physCellId | Physical cell Id of NR Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.8.3.3-8: UEAssistanceInformation (step 14, Table 8.1.5.10.8.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-30 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEAssistanceInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueAssistanceInformation SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| overheatingAssistance | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| idc-Assistance-r16 SEQUENCE { |  |  |  |
| affectedCarrierFreqList-r16 SEQUENCE { | 1 entry |  |  |
| AffectedCarrierFreq-r16 SEQUENCE { |  |  |  |
| carrierFreq-r16 | ARFCN-ValueNR of NR Cell 1 |  |  |
| interferenceDirection-r16 | nr |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.1.5.10.9	UE Assistance Information / IDC / FDM assistance configuration

8.1.5.10.9.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE configured to provide IDC assistance information based on idc-FDM-AssistanceConfig and UE did not transmit a UEAssistanceInformation message with idc-FDM-Assistance and UE is experiencing IDC problems that it cannot solve by itself }

then { The UE transmits a UEAssistanceInformation message to provide IDC assistance information }

}

(2)

with { UE in NR RRC_CONNECTED state and UE already transmits a UEAssistanceInformation message with idc-FDM-Assistance }

ensure that {

when { UE configured to provide IDC assistance information based on idc-FDM-AssistanceConfig and the current idc-FDM-Assistance information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message and UE is experiencing IDC problems that it cannot solve by itself }

then { The UE transmits a UEAssistanceInformation message to provide IDC assistance information }

}

8.1.5.10.9.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in TS 38.331, clause 5.3.5.9, 5.7.4.2 and 5.4.7.3. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.3.5.9]

……

1>	if the received otherConfig includes the idc-AssistanceConfig:

2>	if idc-AssistanceConfig is set to setup:

3>	consider itself to be configured to provide IDC assistance information in accordance with 5.7.4;

2>	else:

3>	consider itself not to be configured to provide IDC assistance information;

…

[TS 38.331, clause 5.7.4.2]

Upon initiating the procedure, the UE shall:

1>	if configured to provide delay budget report:

…

1>	if configured to provide IDC assistance information based on idc-FDM-AssistanceConfig included in idc-AssistanceConfig of a cell group:

2>	if the UE did not transmit a UEAssistanceInformation message with idc-FDM-Assistance since it was configured to provide IDC assistance information:

3>	if on one or more frequency ranges included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself; or

3>	if on one or more supported UL CA or NR-DC combination comprising of frequency ranges included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information;

2>	else if the current idc-FDM-Assistance information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information;

……

[TS 38.331, clause 5.7.4.3]

The UE shall set the contents of the UEAssistanceInformation message as follows:

……

1>	if transmission of the UEAssistanceInformation message is initiated to provide IDC assistance information according to 5.7.4.2 or 5.3.5.3:

……

2>	if there is at least one affected frequency range overlapping with one candidate frequency range included in candidateServingFreqRangeListNR, and the center frequency of the affected frequency range is within the candidate frequency range included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself:

3>	include the field affectedCarrierFreqRangeList with an entry for each affected frequency range;

3>	for each affected frequency range included in the field affectedCarrierFreqRangeList, include centerFreq and affectedBandwidth;

3>	for each affected frequency range included in the field affectedCarrierFreqRangeList, include interferenceDirection and optionally victimSystemType, and set it accordingly;

2>	if there is at least one supported UL CA or NR-DC combinations comprising of candidate frequency ranges included in candidateServingFreqRangeListNR, and each affected frequency range in the UL CA or NR-DC combination overlapping with one candidate frequency range included in candidateServingFreqRangeListNR, and the center frequency of the affected frequency range is within the candidate frequency range included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself:

3>	include the field affectedCarrierFreqRangeCombList with an entry for each supported UL CA or NR-DC combination comprising of frequency ranges that is affected by IDC problems;

3>	for each affected frequency range included in the field affectedCarrierFreqRangeCombList, include centerFreq and affectedBandwidth;

3>	for each UL CA or NR-DC combination included in the field affectedCarrierFreqRangeCombList, include interferenceDirection and optionally victimSystemType, and set it accordingly;

…

8.1.5.10.9.3	Test description

8.1.5.10.9.3.1	Pre-test conditions

System Simulator:

-	NR Cell 14/NRf4 (2390-2400MHz for band n40) and WLAN AP 1 (Cell 27) as specified in TS 38.508-1[4] clause 4.13.

-	NR Cell 10/NRf5 (2496-2556MHz for band n41) and WLAN AP 2 (Cell 27b) as specified in TS 38.508-1[4] clause 4.13.

UE:

-	UE’s WLAN is on.

Preamble:

-	The UE is in state 3N-A on NR Cell 14 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

8.1.5.10.9.3.2	Test procedure sequence

Table 8.1.5.10.9.3.2-1 for FR1 illustrate the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. The configuration "T0" indicates the initial conditions. Subsequent configurations marked "T1", "T2" and "T3" are applied at the points indicated in the Main behaviour description in Table 8.1.5.10.9.3.2-2. The configuration "C3" for "T0", "C4" for "T1", "C5" for "T2" and "C6" for "T3" are specified in TS 38.508-1 [4] Table 4.13-2.

Table 8.1.5.10.9.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 14 | WLAN AP 1 (Cell 27) | NR Cell 10 | WLAN AP 2 (Cell 27b) | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | Refer to “C3” in TS 38.508-1 [4] |  |  |  |  |  |  |
| T1 | Refer to “C4” in TS 38.508-1 [4] |  |  |  |  |  |  |
| T2 | Refer to “C5” in TS 38.508-1 [4] |  |  |  |  |  |  |
| T3 | Refer to “C6” in TS 38.508-1 [4] |  |  |  |  |  |  |

Table 8.1.5.10.9.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a RRCReconfiguration message including OtherConfig with idc-FDM-AssistanceConfig-r18 including CandidateServingFreqRangeListNR-r18 including center frequency and bandwidth for NR Cell 14 and NR Cell 10. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | The SS re-adjusts the power levels according to row "T1" in Table 8.1.5.10.9.3.2-1. | - | - | - | - |
| 4 | The UE associates with the WLAN AP1 and obtains the local IP address of WLAN AP1. Note: The UL signal to WLAN AP shall generate IDC problem. The UE shall detect IDC problem with “T1” in RRC_CONNECTED in NR Cell 14. | - | - | - | - |
| 5 | Check: Does the UE transmit an UEAssistanceInformation message including idc-FDM-Assistance-r18 with at least one AffectedCarrierFreqRange-r18 entry with centerFreq-r18 and affectedBandwidth-r18 information? | --> | NR RRC: UEAssistanceInformation | 1 | P |
| 6 | The SS re-adjusts the power levels according to row "T2" in Table 8.1.5.10.9.3.2-1. | - | - | - | - |
| 7 | The SS transmits an RRCReconfiguration message including to order the UE to perform inter-frequency handover to NR Cell 10. | <-- | NR RRC: RRCReconfiguration | - | - |
| 8 | The UE transmit RRCReconfigurationComplete message in NR Cell 10. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 8A | The SS re-adjusts the power levels according to row "T3" in Table 8.1.5.10.9.3.2-1. | - | - | - | - |
| 9 | The UE associates with the WLAN AP2 and obtains the local IP address of WLAN AP2. Note: The UL signal to WLAN AP shall generate IDC problem. The UE shall detect IDC problem with “T3” in RRC_CONNECTED in NR Cell 10. | - | - | - | - |
| 10 | Check: Does the UE transmit an UEAssistanceInformation message including idc-FDM-Assistance-r18 with at least one AffectedCarrierFreqRange-r18 entry with centerFreq-r18 and affectedBandwidth-r18 information different from the information in Step 5? | --> | NR RRC: UEAssistanceInformation | 2 | P |

8.1.5.10.9.3.3	Specific message contents

Table 8.1.5.10.9.3.3-1: RRCReconfiguration (step 1, Table 8.1.5.10.9.3.2-2)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtensionotherConfig-v1610 SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| otherConfig-v1800 SEQUENCE { |  |  |  |  |  |  |  |
| idc-AssistanceConfig-v1800 CHOICE { |  |  |  |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |  |  |  |
| idc-FDM-AssistanceConfig-r18 CHOICE { |  |  |  |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |  |  |  |
| candidateServingFreqRangeListNR-r18 SEQUENCE (SIZE (1..maxFreqIDC-r16)) OF CandidateServingFreqRangeNR-r18 { |  | 2 entries |  |  |  |  |  |
| CandidateServingFreqRangeNR-r18[1] SEQUENCE { |  |  |  |  |  |  |  |
| candidateCenterFreq-r18 |  | ARFCN-ValueNR of NR Cell 14 |  |  |  |  |  |
| candidateBandwidth-r18 |  | mhz10 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| CandidateServingFreqRangeNR-r18[2] SEQUENCE { |  |  |  |  |  |  |  |
| candidateCenterFreq-r18 |  | ARFCN-ValueNR of NR Cell 10 |  |  |  |  |  |
| candidateBandwidth-r18 |  | mhz60 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 8.1.5.10.9.3.3-2: UEAssistanceInformation (step 5, Table 8.1.5.10.9.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-30 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEAssistanceInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueAssistanceInformation SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| overheatingAssistance | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| idc-FDM-Assistance-r18 SEQUENCE { |  |  |  |
| affectedCarrierFreqRangeList-r18 SEQUENCE (SIZE (1..maxFreqIDC-r16)) OF AffectedCarrierFreqRange-r18 { | 1 entry |  |  |
| AffectedCarrierFreqRange-r18 SEQUENCE { |  |  |  |
| affectedFreqRange-r18 SEQUENCE { |  |  |  |
| centerFreq-r18 | Present |  |  |
| affectedBandwidth-r18 | Present |  |  |
| } |  |  |  |
| interferenceDirection-r18 | nr |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.9.3.3-3: RRCReconfiguration (step 7, Table 8.1.5.10.9.3.2-2)

| Derivation Path: TS 38.508-1 [4] 4.8.1-1A with condition RBConfig_KeyChange |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.9.3.3-4: CellGroupConfig (Table 8.1.5.10.9.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with condition PCell_change and CFRA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| spCellConfigCommon SEQUENCE { |  |  |  |
| physCellId | Physical cell Id of NR Cell 10 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.9.3.3-5: UEAssistanceInformation (step 10, Table 8.1.5.10.9.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-30 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEAssistanceInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueAssistanceInformation SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| overheatingAssistance | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| idc-FDM-Assistance-r18 SEQUENCE { |  |  |  |
| affectedCarrierFreqRangeList-r18 SEQUENCE (SIZE (1..maxFreqIDC-r16)) OF AffectedCarrierFreqRange-r18 { | 1 entry |  |  |
| AffectedCarrierFreqRange-r18 SEQUENCE { |  |  |  |
| affectedFreqRange-r18 SEQUENCE { |  |  |  |
| centerFreq-r18 | Present |  |  |
| affectedBandwidth-r18 | Present |  |  |
| } |  |  |  |
| interferenceDirection-r18 | nr |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.1.5.10.10	UE Assistance Information / IDC / TDM assistance configuration

8.1.5.10.10.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE configured to provide IDC assistance information based on idc-TDM-AssistanceConfig and UE did not transmit a UEAssistanceInformation message with idc-TDM-Assistance and UE is experiencing IDC problems that it cannot solve by itself }

then { The UE transmits a UEAssistanceInformation message to provide IDC assistance information }

}

(2)

with { UE in NR RRC_CONNECTED state and UE already transmits a UEAssistanceInformation message with idc-TDM-Assistance }

ensure that {

when { UE configured to provide IDC assistance information based on idc-TDM-AssistanceConfig and the current idc-TDM-Assistance information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message and UE is experiencing IDC problems that it cannot solve by itself }

then { The UE transmits a UEAssistanceInformation message to provide IDC assistance information }

}

8.1.5.10.10.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in TS 38.331, clause 5.7.4.2 and 5.4.7.3. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.7.4.2]

Upon initiating the procedure, the UE shall:

1>	if configured to provide delay budget report:

…

1>	if configured to provide IDC assistance information based on idc-FDM-AssistanceConfig included in idc-AssistanceConfig of a cell group:

2>	if the UE did not transmit a UEAssistanceInformation message with idc-FDM-Assistance since it was configured to provide IDC assistance information:

3>	if on one or more frequency ranges included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself; or

3>	if on one or more supported UL CA or NR-DC combination comprising of frequency ranges included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information;

2>	else if the current idc-FDM-Assistance information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information;

1>	if configured to provide IDC assistance information based on idc-TDM-AssistanceConfig included in idc-AssistanceConfig of a cell group:

2>	if the UE did not transmit a UEAssistanceInformation message with idc-TDM-Assistance since it was configured to provide IDC assistance information:

3>	if on one or more frequencies included in candidateServingFreqListNR or frequency ranges included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself; or

3>	if on one or more supported UL CA or NR-DC combination comprising of carrier frequencies included in candidateServingFreqListNR or frequency ranges included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information;

2>	else if the current idc-TDM-Assistance information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information;

…

[TS 38.331, clause 5.7.4.3]

The UE shall set the contents of the UEAssistanceInformation message as follows:

…

1>	if transmission of the UEAssistanceInformation message is initiated to provide IDC assistance information according to 5.7.4.2 or 5.3.5.3:

…

2>	if there is at least one affected frequency range overlapping with one candidate frequency range included in candidateServingFreqRangeListNR, and the center frequency of the affected frequency range is within the candidate frequency range included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself:

3>	include the field affectedCarrierFreqRangeList with an entry for each affected frequency range;

3>	for each affected frequency range included in the field affectedCarrierFreqRangeList, include centerFreq and affectedBandwidth;

3>	for each affected frequency range included in the field affectedCarrierFreqRangeList, include interferenceDirection and optionally victimSystemType, and set it accordingly;

2>	if there is at least one supported UL CA or NR-DC combinations comprising of candidate frequency ranges included in candidateServingFreqRangeListNR, and each affected frequency range in the UL CA or NR-DC combination overlapping with one candidate frequency range included in candidateServingFreqRangeListNR, and the center frequency of the affected frequency range is within the candidate frequency range included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself:

3>	include the field affectedCarrierFreqRangeCombList with an entry for each supported UL CA or NR-DC combination comprising of frequency ranges that is affected by IDC problems;

3>	for each affected frequency range included in the field affectedCarrierFreqRangeCombList, include centerFreq and affectedBandwidth;

3>	for each UL CA or NR-DC combination included in the field affectedCarrierFreqRangeCombList, include interferenceDirection and optionally victimSystemType, and set it accordingly;

2>	if there is at least one candidate carrier frequency included in candidateServingFreqListNR or candidate frequency range included in candidateServingFreqRangeListNR or one supported UL CA or NR-DC combination comprising of candidate carrier frequencies included in candidateServingFreqListNR or candidate frequency ranges included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself, and affectedCarrierFreqList or affectedCarrierFreqCombList or affectedCarrierFreqRangeList or affectedCarrierFreqRangeCombList is included, and idc-TDM-AssistanceConfig is set to setup:

3>	include Time Domain Multiplexing (TDM) based assistance information as indicated by idc-TDM-Assistance that could be used to resolve the IDC problems;

…

8.1.5.10.10.3	Test description

8.1.5.10.10.3.1	Pre-test conditions

System Simulator:

-	NR Cell 14/NRf4 (2390-2400MHz for band n40) and WLAN AP 1 (Cell 27) as specified in TS 38.508-1[4] clause 4.13.

-	NR Cell 10/NRf5 (2496-2556MHz for band n41) and WLAN AP 2 (Cell 27b) as specified in TS 38.508-1[4] clause 4.13.

UE:

-	UE’s WLAN is on.

Preamble:

-	The UE is in state 3N-A on NR Cell 14 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

8.1.5.10.10.3.2	Test procedure sequence

Table 8.1.5.10.10.3.2-1 for FR1 illustrate the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. The configuration "T0" indicates the initial conditions. Subsequent configurations marked "T1", “T2” and "T3" are applied at the points indicated in the Main behaviour description in Table 8.1.5.10.10.3.2-2. The configuration "C3" for "T0", "C4" for "T1", "C5" for "T2" and "C6" for "T3" are specified in TS 38.508-1 [4] Table 4.13-2.

Table 8.1.5.10.10.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 14 | WLAN AP 1 (Cell 27) | NR Cell 10 | WLAN AP 2 (Cell 27b) | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | Refer to “C3” in TS 38.508-1 [4] |  |  |  |  |  |  |
| T1 | Refer to “C4” in TS 38.508-1 [4] |  |  |  |  |  |  |
| T2 | Refer to “C5” in TS 38.508-1 [4] |  |  |  |  |  |  |
| T3 | Refer to “C6” in TS 38.508-1 [4] |  |  |  |  |  |  |

Table 8.1.5.10.10.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a RRCReconfiguration message including OtherConfig with idc-FDM-AssistanceConfig-r18 including CandidateServingFreqRangeListNR-r18 including center frequency and bandwidth for NR Cell 14 and NR Cell 10, and idc-TDM-AssistanceConfig-r18. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | The SS re-adjusts the power levels according to row "T1" in Table 8.1.5.10.10.3.2-1. | - | - | - | - |
| 4 | The UE associates with the WLAN AP1 and obtains the local IP address of WLAN AP1. Note: The UL signal to WLAN AP shall generate IDC problem. The UE shall detect IDC problem with “T1” in RRC_CONNECTED in NR Cell 14. | - | - | - | - |
| 5 | Check: Does the UE transmit an UEAssistanceInformation message including idc-TDM-Assistance-r18? | --> | NR RRC: UEAssistanceInformation | 1 | P |
| 6 | The SS re-adjusts the power levels according to row "T2" in Table 8.1.5.10.10.3.2-1. | - | - | - | - |
| 7 | The SS transmits an RRCReconfiguration message including to order the UE to perform inter-frequency handover to NR Cell 10. | <-- | NR RRC: RRCReconfiguration | - | - |
| 8 | The UE transmit RRCReconfigurationComplete message in NR Cell 10. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 9 | The SS re-adjusts the power levels according to row "T3" in Table 8.1.5.10.10.3.2-1. | - | - | - | - |
| 10 | The UE associates with the WLAN AP2 and obtains the local IP address of WLAN AP2. Note: The UL signal to WLAN AP shall generate IDC problem. The UE shall detect IDC problem with “T3” in RRC_CONNECTED in NR Cell 10. | - | - | - | - |
| 11 | Check: Does the UE transmit an UEAssistanceInformation message including idc-TDM-Assistance-r18, which information is different from the information in Step 5? | --> | NR RRC: UEAssistanceInformation | 2 | P |

8.1.5.10.10.3.3	Specific message contents

Table 8.1.5.10.10.3.3-1: RRCReconfiguration (step 1, Table 8.1.5.10.10.3.2-2)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtensionotherConfig-v1610 SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| otherConfig-v1800 SEQUENCE { |  |  |  |  |  |  |  |
| idc-AssistanceConfig-v1800 CHOICE { |  |  |  |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |  |  |  |
| idc-FDM-AssistanceConfig-r18 CHOICE { |  |  |  |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |  |  |  |
| candidateServingFreqRangeListNR-r18 SEQUENCE (SIZE (1..maxFreqIDC-r16)) OF CandidateServingFreqRangeNR-r18 { |  | 2 entries |  |  |  |  |  |
| CandidateServingFreqRangeNR-r18[1] SEQUENCE { |  |  |  |  |  |  |  |
| candidateCenterFreq-r18 |  | ARFCN-ValueNR of NR Cell 14 |  |  |  |  |  |
| candidateBandwidth-r18 |  | mhz10 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| CandidateServingFreqRangeNR-r18[2] SEQUENCE { |  |  |  |  |  |  |  |
| candidateCenterFreq-r18 |  | ARFCN-ValueNR of NR Cell 10 |  |  |  |  |  |
| candidateBandwidth-r18 |  | mhz60 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| idc-TDM-AssistanceConfig-r18 |  | setup |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 8.1.5.10.10.3.3-2: UEAssistanceInformation (step 5, Table 8.1.5.10.10.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-30 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEAssistanceInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueAssistanceInformation SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| overheatingAssistance | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| idc-FDM-Assistance-r18 SEQUENCE { |  |  |  |
| affectedCarrierFreqRangeList-r18 SEQUENCE (SIZE (1..maxFreqIDC-r16)) OF AffectedCarrierFreqRange-r18 { | 1 entry |  |  |
| AffectedCarrierFreqRange-r18 SEQUENCE { |  |  |  |
| affectedFreqRange-r18 SEQUENCE { |  |  |  |
| centerFreq-r18 | Present |  |  |
| affectedBandwidth-r18 | Present |  |  |
| } |  |  |  |
| interferenceDirection-r18 | nr |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| idc-TDM-Assistance-r18 SEQUENCE { |  | The actual IE values depend on UE reporting |  |
| cycleLength-r18 | Present |  |  |
| startOffset-r18 | Present |  |  |
| slotOffset-r18 | Present |  |  |
| activeDuration-r18 | Present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.10.3.3-3: RRCReconfiguration (step 7, Table 8.1.5.10.10.3.2-2)

| Derivation Path: TS 38.508-1 [4] 4.8.1-1A with condition RBConfig_KeyChange |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.10.3.3-4: CellGroupConfig (Table 8.1.5.10.10.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with condition PCell_change and CFRA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| spCellConfigCommon SEQUENCE { |  |  |  |
| physCellId | Physical cell Id of NR Cell 10 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.10.3.3-5: UEAssistanceInformation (step 11, Table 8.1.5.10.10.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-30 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEAssistanceInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueAssistanceInformation SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| overheatingAssistance | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| idc-FDM-Assistance-r18 SEQUENCE { |  |  |  |
| affectedCarrierFreqRangeList-r18 SEQUENCE (SIZE (1..maxFreqIDC-r16)) OF AffectedCarrierFreqRange-r18 { | 1 entry |  |  |
| AffectedCarrierFreqRange-r18 SEQUENCE { |  |  |  |
| affectedFreqRange-r18 SEQUENCE { |  |  |  |
| centerFreq-r18 | Present |  |  |
| affectedBandwidth-r18 | Present |  |  |
| } |  |  |  |
| interferenceDirection-r18 | nr |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| idc-TDM-Assistance-r18 SEQUENCE { |  | This entry only exists when at least one IE value is different from the IE values in step 5. Otherwise, this entry will not exist. |  |
| cycleLength-r18 | Present |  |  |
| startOffset-r18 | Present |  |  |
| slotOffset-r18 | Present |  |  |
| activeDuration-r18 | Present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.1.5.10.11	UE Assistance Information / IDC / FDM assistance configuration / Inter-band CA

8.1.5.10.11.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE configured to provide IDC assistance information based on idc-FDM-AssistanceConfig and UE did not transmit a UEAssistanceInformation message with idc-FDM-Assistance and UE is experiencing IDC problems that it cannot solve by itself }

then { The UE transmits a UEAssistanceInformation message to provide IDC assistance information including AffectedCarrierFreqRangeCombList-r18 }

}

8.1.5.10.11.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in TS 38.331, clauses 5.7.4.2 and 5.4.7.3. Unless otherwise stated these are Rel-18 requirements.

[TS 38.331, clause 5.7.4.2]

Upon initiating the procedure, the UE shall:

1>	if configured to provide delay budget report:

…

1>	if configured to provide IDC assistance information based on idc-FDM-AssistanceConfig included in idc-AssistanceConfig of a cell group:

2>	if the UE did not transmit a UEAssistanceInformation message with idc-FDM-Assistance since it was configured to provide IDC assistance information:

3>	if on one or more frequency ranges included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself; or

3>	if on one or more supported UL CA or NR-DC combination comprising of frequency ranges included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information;

2>	else if the current idc-FDM-Assistance information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information;

1>	if configured to provide IDC assistance information based on idc-TDM-AssistanceConfig included in idc-AssistanceConfig of a cell group:

2>	if the UE did not transmit a UEAssistanceInformation message with idc-TDM-Assistance since it was configured to provide IDC assistance information:

3>	if on one or more frequencies included in candidateServingFreqListNR or frequency ranges included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself; or

3>	if on one or more supported UL CA or NR-DC combination comprising of carrier frequencies included in candidateServingFreqListNR or frequency ranges included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information;

2>	else if the current idc-TDM-Assistance information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information;

…

[TS 38.331, clause 5.7.4.3]

The UE shall set the contents of the UEAssistanceInformation message as follows:

…

1>	if transmission of the UEAssistanceInformation message is initiated to provide IDC assistance information according to 5.7.4.2 or 5.3.5.3:

…

2>	if there is at least one affected frequency range overlapping with one candidate frequency range included in candidateServingFreqRangeListNR, and the center frequency of the affected frequency range is within the candidate frequency range included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself:

3>	include the field affectedCarrierFreqRangeList with an entry for each affected frequency range;

3>	for each affected frequency range included in the field affectedCarrierFreqRangeList, include centerFreq and affectedBandwidth;

3>	for each affected frequency range included in the field affectedCarrierFreqRangeList, include interferenceDirection and optionally victimSystemType, and set it accordingly;

2>	if there is at least one supported UL CA or NR-DC combinations comprising of candidate frequency ranges included in candidateServingFreqRangeListNR, and each affected frequency range in the UL CA or NR-DC combination overlapping with one candidate frequency range included in candidateServingFreqRangeListNR, and the center frequency of the affected frequency range is within the candidate frequency range included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself:

3>	include the field affectedCarrierFreqRangeCombList with an entry for each supported UL CA or NR-DC combination comprising of frequency ranges that is affected by IDC problems;

3>	for each affected frequency range included in the field affectedCarrierFreqRangeCombList, include centerFreq and affectedBandwidth;

3>	for each UL CA or NR-DC combination included in the field affectedCarrierFreqRangeCombList, include interferenceDirection and optionally victimSystemType, and set it accordingly;

2>	if there is at least one candidate carrier frequency included in candidateServingFreqListNR or candidate frequency range included in candidateServingFreqRangeListNR or one supported UL CA or NR-DC combination comprising of candidate carrier frequencies included in candidateServingFreqListNR or candidate frequency ranges included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself, and affectedCarrierFreqList or affectedCarrierFreqCombList or affectedCarrierFreqRangeList or affectedCarrierFreqRangeCombList is included, and idc-TDM-AssistanceConfig is set to setup:

3>	include Time Domain Multiplexing (TDM) based assistance information as indicated by idc-TDM-Assistance that could be used to resolve the IDC problems;

8.1.5.10.11.3	Test description

8.1.5.10.11.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 with band n3 and NR Cell 10 with band n8.

-	WLAN AP1 (Cell 27) with Channel 1 (2401-2423MHz).

-	NR Cell 1 is the PCell, and NR Cell 10 is the SCell.

-	System information combination NR-4 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

-	UE’s WLAN is on.

Preamble:

-	The UE is in state 3N-A, and with UE test loop mode B active if pc_IP_Ping is set to FALSE, as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1.

8.1.5.10.11.3.2	Test procedure sequence

Table 8.1.5.10.11.3.2-1 for FR1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. The configuration "T0" indicates the initial conditions. Subsequent configurations marked "T1" and "T2" are applied at the points indicated in the main behaviour description in Table 8.1.5.10.11.3.2-2. The configuration "C7" for "T0", "C8" for "T1" and "C9" for "T2" are specified in TS 38.508-1 [4] Table 4.13-3.

Table 8.1.5.10.11.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 10 | WLAN AP 1 (Cell 27) | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Refer to “C7” in TS 38.508-1 [4] |  |  |  |  |  |
| T1 | Refer to “C8” in TS 38.508-1 [4] |  |  |  |  |  |
| T2 | Refer to “C9” in TS 38.508-1 [4] |  |  |  |  |  |

Table 8.1.5.10.11.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS re-adjusts the power levels according to row "T1" in Table 8.1.5.10.11.3.2-1. | - | - | - | - |
| 2 | The SS transmits an RRCReconfigurationmessage containing an sCellToAddModList with SCell NR Cell 10 addition. | <-- | NR RRC: RRCReconfiguration | - | - |
| 3 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 4 | The SS transmits an RRCReconfiguration message including OtherConfig with idc-FDM-AssistanceConfig-r18 including CandidateServingFreqRangeListNR-r18 for NR Cell 1, and NR Cell 10. | <-- | NR RRC: RRCReconfiguration | - | - |
| 5 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 6 | The SS re-adjusts the power levels according to row "T2" in Table 8.1.5.10.11.3.2-1. | - | - | - | - |
| 7 | The test procedure in TS 38.508-1 subclause 4.9.1 is performed to enable the UE exchanging IP data on DRB1 and Cell 10. Note: The CA inter-modulation affected the frequency of WLAN AP 1 with “T2”. | - | - | - | - |
| 8 | Check: Does the UE transmit a UEAssistanceInformation message including idc-FDM-Assistance-r18 on NR Cell 1? | --> | NR RRC: UEAssistanceInformation | 1 | P |

8.1.5.10.11.3.3	Specific message contents

Table 8.1.5.10.11.3.3-1: RRCReconfiguration (step 2, Table 8.1.5.10.11.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.11.3.3-2: CellGroupConfig (Table 8.1.5.10.11.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with Condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig { | 1 entry |  |  |
| SCellConfig[1] SEQUENCE { |  | entry 1 |  |
| sCellConfigCommon SEQUENCE { |  |  |  |
| physCellId | Physical Cell Identity of NR Cell 10 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.11.3.3-3: RRCReconfiguration (step 4, Table 8.1.5.10.11.3.2-2)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| otherConfig-v1800 SEQUENCE { |  |  |  |
| idc-AssistanceConfig-v1800 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| idc-FDM-AssistanceConfig-r18 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| candidateServingFreqRange ListNR-r18 SEQUENCE (SIZE (1..maxFreqIDC-r16)) OF CandidateServingFreqRangeNR-r18 { | 2 entries |  |  |
| CandidateServingFreq RangeNR-r18[1] SEQUENCE { |  | entry 1 |  |
| candidateCenterFreq-r18 | ARFCN-ValueNR of NR Cell 1 |  |  |
| candidateBandwidth-r18 | mhz10 |  |  |
| } |  |  |  |
| CandidateServingFreq RangeNR-r18[2] SEQUENCE { |  | entry 2 |  |
| candidateCenterFreq-r18 | ARFCN-ValueNR of NR Cell 10 |  |  |
| candidateBandwidth-r18 | mhz60 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.11.3.3-4: UEAssistanceInformation (step 8, Table 8.1.5.10.11.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-30 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEAssistanceInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueAssistanceInformation SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| overheatingAssistance | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| idc-FDM-Assistance-r18 SEQUENCE { |  |  |  |
| affectedCarrierFreqRangeList-r18 | Not checked |  |  |
| affectedCarrierFreqRangeComb List-r18 SEQUENCE (SIZE (1..maxCombIDC-r16)) OF AffectedCarrierFreqRangeComb-r18 { | 1 entry |  |  |
| AffectedCarrierFreqRangeComb-r18 SEQUENCE (SIZE (2..maxNrofServingCells)) OF AffectedFreqRange-r18 { | 2 entries |  |  |
| affectedFreqRange-r18 [1] SEQUENCE { |  | The value of this entry is in the candidate frequency range of NR Cell 1 |  |
| centerFreq-r18 | Present but contents not checked |  |  |
| affectedBandwidth-r18 | Present but contents not checked |  |  |
| } |  |  |  |
| affectedFreqRange-r18 [2] SEQUENCE { |  | The value of this entry is in the candidate frequency range of NR Cell 10 |  |
| centerFreq-r18 | Present but contents not checked |  |  |
| affectedBandwidth-r18 | Present but contents not checked |  |  |
| } |  |  |  |
| interferenceDirection-r18 | other |  |  |
| victimSystemType-r18 SEQUENCE { |  |  |  |
| wlan-r16 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.1.5.10.12	UE Assistance Information / IDC / TDM assistance configuration / Inter-band CA

8.1.5.10.12.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE configured to provide IDC assistance information based on idc-TDM-AssistanceConfig and UE did not transmit a UEAssistanceInformation message with idc-TDM-Assistance and UE is experiencing IDC problems that it cannot solve by itself }

then { The UE transmits a UEAssistanceInformation message to provide IDC assistance information, including AffectedCarrierFreqRangeCombList-r18 }

}

8.1.5.10.12.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in TS 38.331, clauses 5.7.4.2 and 5.4.7.3. Unless otherwise stated these are Rel-18 requirements.

[TS 38.331, clause 5.7.4.2]

Upon initiating the procedure, the UE shall:

1>	if configured to provide delay budget report:

…

1>	if configured to provide IDC assistance information based on idc-FDM-AssistanceConfig included in idc-AssistanceConfig of a cell group:

2>	if the UE did not transmit a UEAssistanceInformation message with idc-FDM-Assistance since it was configured to provide IDC assistance information:

3>	if on one or more frequency ranges included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself; or

3>	if on one or more supported UL CA or NR-DC combination comprising of frequency ranges included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information;

2>	else if the current idc-FDM-Assistance information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information;

1>	if configured to provide IDC assistance information based on idc-TDM-AssistanceConfig included in idc-AssistanceConfig of a cell group:

2>	if the UE did not transmit a UEAssistanceInformation message with idc-TDM-Assistance since it was configured to provide IDC assistance information:

3>	if on one or more frequencies included in candidateServingFreqListNR or frequency ranges included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself; or

3>	if on one or more supported UL CA or NR-DC combination comprising of carrier frequencies included in candidateServingFreqListNR or frequency ranges included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information;

2>	else if the current idc-TDM-Assistance information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information;

…

[TS 38.331, clause 5.7.4.3]

The UE shall set the contents of the UEAssistanceInformation message as follows:

…

1>	if transmission of the UEAssistanceInformation message is initiated to provide IDC assistance information according to 5.7.4.2 or 5.3.5.3:

…

2>	if there is at least one affected frequency range overlapping with one candidate frequency range included in candidateServingFreqRangeListNR, and the center frequency of the affected frequency range is within the candidate frequency range included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself:

3>	include the field affectedCarrierFreqRangeList with an entry for each affected frequency range;

3>	for each affected frequency range included in the field affectedCarrierFreqRangeList, include centerFreq and affectedBandwidth;

3>	for each affected frequency range included in the field affectedCarrierFreqRangeList, include interferenceDirection and optionally victimSystemType, and set it accordingly;

2>	if there is at least one supported UL CA or NR-DC combinations comprising of candidate frequency ranges included in candidateServingFreqRangeListNR, and each affected frequency range in the UL CA or NR-DC combination overlapping with one candidate frequency range included in candidateServingFreqRangeListNR, and the center frequency of the affected frequency range is within the candidate frequency range included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself:

3>	include the field affectedCarrierFreqRangeCombList with an entry for each supported UL CA or NR-DC combination comprising of frequency ranges that is affected by IDC problems;

3>	for each affected frequency range included in the field affectedCarrierFreqRangeCombList, include centerFreq and affectedBandwidth;

3>	for each UL CA or NR-DC combination included in the field affectedCarrierFreqRangeCombList, include interferenceDirection and optionally victimSystemType, and set it accordingly;

2>	if there is at least one candidate carrier frequency included in candidateServingFreqListNR or candidate frequency range included in candidateServingFreqRangeListNR or one supported UL CA or NR-DC combination comprising of candidate carrier frequencies included in candidateServingFreqListNR or candidate frequency ranges included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself, and affectedCarrierFreqList or affectedCarrierFreqCombList or affectedCarrierFreqRangeList or affectedCarrierFreqRangeCombList is included, and idc-TDM-AssistanceConfig is set to setup:

3>	include Time Domain Multiplexing (TDM) based assistance information as indicated by idc-TDM-Assistance that could be used to resolve the IDC problems;

8.1.5.10.12.3	Test description

8.1.5.10.12.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 with band n3 and NR Cell 10 with band n8.

-	WLAN AP1 (Cell 27) with Channel 1 (2401-2423MHz).

-	NR Cell 1 is the PCell, and NR Cell 10 is the SCell.

-	System information combination NR-4 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

-	UE’s WLAN is on.

Preamble:

-	The UE is in state 3N-A, and with UE test loop mode B active if pc_IP_Ping is set to FALSE, as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1.

8.1.5.10.12.3.2	Test procedure sequence

Table 8.1.5.10.12.3.2-1 for FR1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. The configuration "T0" indicates the initial conditions. Subsequent configurations marked "T1" and "T2" are applied at the points indicated in the main behaviour description in Table 8.1.5.10.12.3.2-2. The configuration "C7" for "T0", "C8" for "T1" and "C9" for "T2" are specified in TS 38.508-1 [4] Table 4.13-3.

Table 8.1.5.10.12.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 10 | WLAN AP 1 (Cell 27) | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Refer to “C7” in TS 38.508-1 [4] |  |  |  |  |  |
| T1 | Refer to “C8” in TS 38.508-1 [4] |  |  |  |  |  |
| T2 | Refer to “C9” in TS 38.508-1 [4] |  |  |  |  |  |

Table 8.1.5.10.12.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS re-adjusts the power levels according to row "T1" in Table 8.1.5.10.12.3.2-1. | - | - | - | - |
| 2 | The SS transmits an RRCReconfigurationmessage containing an sCellToAddModList with SCell NR Cell 10 addition. | <-- | NR RRC: RRCReconfiguration | - | - |
| 3 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 4 | The SS transmits an RRCReconfiguration message including OtherConfig with idc-FDM-AssistanceConfig-r18 including CandidateServingFreqRangeListNR-r18 for NR Cell 1, and NR Cell 10, and idc-TDM-AssistanceConfig-r18. | <-- | NR RRC: RRCReconfiguration | - | - |
| 5 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 6 | The SS re-adjusts the power levels according to row "T2" in Table 8.1.5.10.12.3.2-1. | - | - | - | - |
| 7 | The test procedure in TS 38.508-1 subclause 4.9.1 is performed to enable the UE exchanging IP data on DRB1 and Cell 10. Note: The CA inter-modulation affected the frequency of WLAN AP 1 with “T2”. | - | - | - | - |
| 8 | Check: Does the UE transmit a UEAssistanceInformation message including idc-TDM-Assistance-r18 on NR Cell 1? | --> | NR RRC: UEAssistanceInformation | 1 | P |

8.1.5.10.12.3.3	Specific message contents

Table 8.1.5.10.12.3.3-1: RRCReconfiguration (step 2, Table 8.1.5.10.12.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.12.3.3-2: CellGroupConfig (Table 8.1.5.10.12.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with Condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig { | 1 entry |  |  |
| SCellConfig[1] SEQUENCE { |  | entry 1 |  |
| sCellConfigCommon SEQUENCE { |  |  |  |
| physCellId | Physical Cell Identity of NR Cell 10 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.12.3.3-3: RRCReconfiguration (step 4, Table 8.1.5.10.12.3.2-2)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| otherConfig-v1800 SEQUENCE { |  |  |  |
| idc-AssistanceConfig-v1800 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| idc-FDM-AssistanceConfig-r18 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| candidateServingFreqRange ListNR-r18 SEQUENCE (SIZE (1..maxFreqIDC-r16)) OF CandidateServingFreqRangeNR-r18 { | 2 entries |  |  |
| CandidateServingFreq RangeNR-r18[1] SEQUENCE { |  | entry 1 |  |
| candidateCenterFreq-r18 | ARFCN-ValueNR of NR Cell 14 |  |  |
| candidateBandwidth-r18 | mhz10 |  |  |
| } |  |  |  |
| CandidateServingFreq RangeNR-r18[2] SEQUENCE { |  | entry 2 |  |
| candidateCenterFreq-r18 | ARFCN-ValueNR of NR Cell 1 |  |  |
| candidateBandwidth-r18 | mhz60 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| idc-TDM-AssistanceConfig-r18 | setup |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.12.3.3-4: UEAssistanceInformation (step 8, Table 8.1.5.10.12.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-30 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEAssistanceInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueAssistanceInformation SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| overheatingAssistance | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| idc-FDM-Assistance-r18 SEQUENCE { |  |  |  |
| affectedCarrierFreqRangeList-r18 | Not checked |  |  |
| affectedCarrierFreqRangeComb List-r18 SEQUENCE (SIZE (1..maxCombIDC-r16)) OF AffectedCarrierFreqRangeComb-r18 { | 1 entry |  |  |
| AffectedCarrierFreqRangeComb-r18 SEQUENCE (SIZE (2..maxNrofServingCells)) OF AffectedFreqRange-r18 { | 2 entries |  |  |
| affectedFreqRange-r18 [1] SEQUENCE { |  | The value of this entry is in the candidate frequency range of NR Cell 1 |  |
| centerFreq-r18 | Present but contents not checked |  |  |
| affectedBandwidth-r18 | Present but contents not checked |  |  |
| } |  |  |  |
| affectedFreqRange-r18 [2] SEQUENCE { |  | The value of this entry is in the candidate frequency range of NR Cell 10 |  |
| centerFreq-r18 | Present but contents not checked |  |  |
| affectedBandwidth-r18 | Present but contents not checked |  |  |
| } |  |  |  |
| interferenceDirection-r18 | other |  |  |
| victimSystemType-r18 SEQUENCE { |  |  |  |
| wlan-r16 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| idc-TDM-Assistance-r18 SEQUENCE { |  | The actual IE values depend on UE reporting |  |
| cycleLength-r18 | Present but contents not checked |  |  |
| startOffset-r18 | Present but contents not checked |  |  |
| slotOffset-r18 | Present but contents not checked |  |  |
| activeDuration-r18 | Present but contents not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.1.5.11	Idle/Inactive measurements

##### 8.1.5.11.1	Idle/Inactive measurements / Idle mode / SIB11 configuration / Measurement of NR cells

8.1.5.11.1.1	Test Purpose (TP)

(1)

with { UE in NR RRC_IDLE state after receiving RRCRelease message with measIdleConfig not including measIdleCarrierListNR while being connected to serving cell in which SIB1 includes idleModeMeasurementsNR and SIB11 is configured with measIdleCarrierListNR with ssb-MeasConfig and UE supporting idleInactiveNR-MeasReport }

ensure that {

when { UE receives UEInformationRequest message with idleModeMeasurementReq after successful measurement procedure and RRC connection re-establishment }

then { UE reports measurement results in UEInformationResponse message }

}

(2)

with { UE in NR RRC_IDLE state after receiving RRCRelease message with measIdleConfig not including measIdleCarrierListNR while being connected to serving cell in which SIB1 includes idleModeMeasurementsNR and SIB11 is configured with measIdleCarrierListNR without ssb-MeasConfig and SIB4 is configured with ssb-MeasConfig parameters and UE supporting idleInactiveNR-MeasReport }

ensure that {

when { UE receives UEInformationRequest message with idleModeMeasurementReq after successful measurement procedure and RRC connection re-establishment }

then { UE reports measurement results in UEInformationResponse message }

}

(3)

with { UE in NR RRC_IDLE state after UE has received RRCRelease message with measIdleConfig not including measIdleCarrierListNR while being connected to serving cell in which SIB1 includes idleModeMeasurementsNR and SIB11 is not configured and UE supporting idleInactiveNR-MeasReport }

ensure that {

when { UE receives UEInformationRequest message with idleModeMeasurementReq after unsuccessful measurement procedure and RRC connection re-establishment }

then { UE does not report measurement results in UEInformationResponse message }

}

8.1.5.11.1.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 38.331 clauses 5.7.8.1, 5.7.8.1a, 5.7.8.2a, 5.3.8.3, 5.7.8.3 and 5.7.10.3. Unless otherwise stated, these are Rel-16 requirements.

[TS 38.331, clause 5.7.8.1]

This procedure specifies the measurements to be performed and stored by a UE in RRC_IDLE and RRC_INACTIVE when it has an idle/inactive measurement configuration.

[TS 38.331, clause 5.7.8.1a]

The purpose of this procedure is to update the idle/inactive measurement configuration.

The UE initiates this procedure while T331 is running and one of the following conditions is met:

1>	upon selecting a cell when entering RRC_IDLE or RRC-INACTIVE from RRC_CONNECTED or RRC_INACTIVE; or

1>	upon update of system information (SIB4, or SIB11), e.g. due to intra-RAT cell (re)selection;

While in RRC_IDLE or RRC_INACTIVE, and T331 is running, the UE shall:

1>	if VarMeasIdleConfig includes neither a measIdleCarrierListEUTRA nor a measIdleCarrierListNR received from the RRCRelease message:

2>	if the UE supports idleInactiveNR-MeasReport:

3>	if SIB11 includes the measIdleConfigSIB and contains measIdleCarrierListNR:

4>	store or replace the measIdleCarrierListNR of measIdleConfigSIB of SIB11 within VarMeasIdleConfig;

3>	else:

4>	remove the measIdleCarrierListNR in VarMeasIdleConfig, if stored;

1>	for each entry in the measIdleCarrierListNR within VarMeasIdleConfig that does not contain an ssb-MeasConfig received from the RRCRelease message:

2>	if there is an entry in measIdleCarrierListNR in measIdleConfigSIB of SIB11 that has the same carrier frequency and subcarrier spacing as the entry in the measIdleCarrierListNR within VarMeasIdleConfig and that contains ssb-MeasConfig:

3>	delete the ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig;

3>	store the SSB measurement configuration from SIB11 into nrofSS-BlocksToAverage, absThreshSS-BlocksConsolidation, smtc, ssb-ToMeasure, deriveSSB-IndexFromCell, and ss-RSSI-Measurement within ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig;

2>	else if there is an entry in interFreqCarrierFreqList of SIB4 with the same carrier frequency and subcarrier spacing as the entry in measIdleCarrierListNR within VarMeasIdleConfig:

3>	delete the ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig;

3>	store the SSB measurement configuration from SIB4 into nrofSS-BlocksToAverage, absThreshSS-BlocksConsolidation, smtc, ssb-ToMeasure, deriveSSB-IndexFromCell, and ss-RSSI-Measurement within ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig;

2>	else:

3>	remove the ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig, if stored;

1>	perform measurements according to 5.7.8.2a.

[TS 38.331, clause 5.7.8.2a]

When performing measurements on NR carriers according to this clause, the UE shall derive the cell quality as specified in 5.5.3.3 and consider the beam quality to be the value of the measurement results of the concerned beam, where each result is averaged as described in TS 38.215 [9].

While in RRC_IDLE or RRC_INACTIVE, and T331 is running, the UE shall:

1>	perform the measurements in accordance with the following:

2>	if the VarMeasIdleConfig includes the measIdleCarrierListNR and the SIB1 contains idleModeMeasurementsNR:

3>	for each entry in measIdleCarrierListNR within VarMeasIdleConfig that contains ssb-MeasConfig:

4>	if UE supports carrier aggregation or NR-DC between serving carrier and the carrier frequency and subcarrier spacing indicated by carrierFreq and ssbSubCarrierSpacing within the corresponding entry:

5>	perform measurements in the carrier frequency and subcarrier spacing indicated by carrierFreq and ssbSubCarrierSpacing within the corresponding entry;

5>	if the reportQuantities is set to rsrq:

6>	consider RSRQ as the cell sorting quantity;

5>	else:

6>	consider RSRP as the cell sorting quantity;

5>	if the measCellListNR is included:

6>	consider cells identified by each entry within the measCellListNR to be applicable for idle/inactive measurement reporting;

5>	else:

6>	consider up to maxCellMeasIdle strongest identified cells, according to the sorting quantity, to be applicable for idle/inactive measurement reporting;

5>	for all cells applicable for idle/inactive measurement reporting, derive cell measurement results for the measurement quantities indicated by reportQuantities;

5>	store the derived cell measurement results as indicated by reportQuantities for cells applicable for idle/inactive measurement reporting within measResultsPerCarrierListIdleNR in the measReportIdleNR in VarMeasIdleReport in decreasing order of the cell sorting quantity, i.e. the best cell is included first, as follows:

6>	if qualityThreshold is configured:

7>	include the measurement results from the cells applicable for idle/inactive measurement reporting whose RSRP/RSRQ measurement results are above the value(s) provided in qualityThreshold;

6>	else:

7>	include the measurement results from all cells applicable for idle/inactive measurement reporting;

5>	if beamMeasConfigIdle is included in the associated entry in measIdleCarrierListNR and if UE supports idleInactiveNR-MeasBeamReport for the FR of the carrier frequency indicated by carrierFreq within the associated entry, for each cell in the measurement results:

6>	derive beam measurements based on SS/PBCH block for each measurement quantity indicated in reportQuantityRS-Indexes, as described in TS 38.215 [9];

6>	if the reportQuantityRS-Indexes is set to rsrq:

7>	consider RSRQ as the beam sorting quantity;

6>	else:

7>	consider RSRP as the beam sorting quantity;

6>	set resultsSSB-Indexes to include up to maxNrofRS-IndexesToReport SS/PBCH block indexes in order of decreasing beam sorting quantity as follows:

7>	include the index associated to the best beam for the sorting quantity and if absThreshSS-BlocksConsolidation is included, the remaining beams whose sorting quantity is above absThreshSS-BlocksConsolidation;

6>	if the includeBeamMeasurements is set to true:

7>	include the beam measurement results as indicated by reportQuantityRS-Indexes;

2>	if, as a result of the procedure in this subclause, the UE performs measurements in one or more carrier frequency indicated by measIdleCarrierListNR or measIdleCarrierListEUTRA:

3>	store the cell measurement results for RSRP and RSRQ for the serving cell within measResultServingCell in the measReportIdleNR in VarMeasIdleReport.

3>	if the VarMeasIdleConfig includes the measIdleCarrierListNR and it contains an entry with carrierFreq set to the value of the serving frequency:

4>	if beamMeasConfigIdle is included in that entry, and if the UE supports idleInactiveNR- MeasBeamReport for the FR of the serving cell:

5>	derive beam measurements based on SS/PBCH block for each measurement quantity indicated in reportQuantityRS-Indexes, as described in TS 38.215 [9];

5>	if the reportQuantityRS-Indexes is set to rsrq:

6>	consider RSRQ as the beam sorting quantity;

5>	else:

6>	consider RSRP as the beam sorting quantity;

5>	set resultsSSB-Indexes to include up to maxNrofRS-IndexesToReport SS/PBCH block indexes in order of decreasing beam sorting quantity as follows:

6>	include the index associated to the best beam for the sorting quantity and if absThreshSS-BlocksConsolidation is included in SIB2 of serving cell, the remaining beams whose sorting quantity is above absThreshSS-BlocksConsolidation;

5>	if the includeBeamMeasurements is set to true:

6>	include the beam measurement results as indicated by reportQuantityRS-Indexes;

NOTE 1:	How the UE performs idle/inactive measurements is up to UE implementation as long as the requirements in TS 38.133 [14] are met for measurement reporting.

NOTE 2:	The UE is not required to perform idle/inactive measurements on a given carrier if the SSB configuration of that carrier provided via dedicated signalling is different from the SSB configuration broadcasted in the serving cell, if any.

NOTE 3:	How the UE prioritizes which frequencies to measure or report (in case it is configured with more frequencies than it can measure or report) is left to UE implementation.

[TS 38.331, clause 5.3.8.3]

The UE shall:

1>	if the RRCRelease includes the measIdleConfig:

2>	if T331 is running:

3> stop timer T331;

3>	perform the actions as specified in 5.7.8.3;

2>	if the measIdleConfig is set to setup:

3>	store the received measIdleDuration in VarMeasIdleConfig;

3>	start timer T331 with the value set to measIdleDuration;

[TS 38.331, clause 5.7.8.3]

The UE shall:

1>	if T331 expires or is stopped:

2>	release the VarMeasIdleConfig.

NOTE:	It is up to UE implementation whether to continue idle/inactive measurements according to SIB11 and SIB4 configurations or according to E-UTRA SIB5 and E-UTRA SIB24 configurations as specified in TS 36.331 [10] upon inter-RAT cell reselection to E-UTRA, after T331 has expired or stopped.

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

1>	if the idleModeMeasurementReq is included in the UEInformationRequest and the UE has stored VarMeasIdleReport that contains measurement information concerning cells other than the PCell:

2>	set the measResultIdleNR in the UEInformationResponse message to the value of measReportIdleNR in the VarMeasIdleReport, if available;

8.1.5.11.1.3	Test Description

8.1.5.11.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1, serving cell. System information combination NR-19 defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used. Condition EMR_NR used for SIB1 and condition EMR_NR_SIB11 used for SIB11.

-	NR Cell 3, suitable for inter-frequency measurements. System information combination NR-1 defined in TS 38.508-1 [4] clause 4.4.3.1.2.

UE:

-	None.

Preamble:

-	The UE is in 5GS state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3 on NR Cell 1.

8.1.5.11.1.3.2	Test procedure sequence

Table 8.1.5.11.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits RRCRelease message including measIdleConfig without measIdleCarrierListNR to the UE. | <-- | NR RRC: RRCRelease | - | - |
| 2 | Wait for 15 s for UE to enter RRC_IDLE mode and to perform measurements. | - | - | - | - |
| 3-10 | Steps 1 to 8 of procedure described in TS 38.508-1 [4] table 4.5.4.2-3: NR RRC_CONNECTED are performed. | - | - | - | - |
| 11 | The SS transmits UEInformationRequest message including idleModeMeasurementReq to the UE. | <-- | NR RRC: UEInformationRequest | - | - |
| 12 | Check: Does the UE transmit UEInformationResponse including measResultIdleNR? | --> | NR RRC: UEInformationResponse | 1 | P |
| 13 | The SS changes system information combination of NR Cell 1 to NR-18 defined in TS 38.508-1 [4] clause 4.4.3.1.2 with condition EMR_NR for SIB1 and EMR_NR_SIB4 for SIB11. | - | - | - | - |
| 14 | The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. | <-- | PDCCH (DCI 1_0): Short Message | - | - |
| 15 | Wait for 2.1* modification period to allow the new system information to take effect. | - | - | - | - |
| 16 | The SS transmits RRCRelease message including measIdleConfig without measIdleCarrierListNR to the UE. | <-- | NR RRC: RRCRelease | - | - |
| 17 | Wait for 15 s for UE to enter RRC_IDLE mode and to perform measurements. | - | - | - | - |
| 18-25 | Steps 1 to 8 of procedure described in TS 38.508-1 [4] table 4.5.4.2-3: NR RRC_CONNECTED are performed. | - | - | - | - |
| 26 | The SS transmits UEInformationRequest message including idleModeMeasurementReq to the UE. | <-- | NR RRC: UEInformationRequest | - | - |
| 27 | Check: Does the UE transmit UEInformationResponse including measResultIdleNR? | --> | NR RRC: UEInformationResponse | 2 | P |
| 28 | The SS changes system information combination of NR Cell 1 to NR-1 to have no configuration for measurement procedure. | - | - | - | - |
| 29 | The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. | <-- | PDCCH (DCI 1_0): Short Message | - | - |
| 30 | Wait for 2.1* modification period to allow the new system information to take effect. | - | - | - | - |
| 31 | The SS transmits RRCRelease message including measIdleConfig without measIdleCarrierListNR to the UE. | <-- | NR RRC: RRCRelease | - | - |
| 32 | Wait for 15 s for UE to enter RRC_IDLE mode and to perform measurements. | - | - | - | - |
| 33-40 | Steps 1 to 8 of procedure described in TS 38.508-1 [4] table 4.5.4.2-3: NR RRC_CONNECTED are performed. | - | - | - | - |
| 41 | The SS transmits UEInformationRequest message with idleModeMeasurementReq to the UE. | <-- | NR RRC: UEInformationRequest | - | - |
| 42 | Check: Does the UE transmit UEInformationResponse not including measResultIdleNR? | --> | NR RRC: UEInformationResponse | 3 | P |

8.1.5.11.1.3.3	Specific message contents

Table 8.1.5.11.1.3.3-1: RRCRelease (steps 1, 16 and 31, Table 8.1.5.11.1.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |
| nonCriticalExtensions SEQUENCE { |  |  |  |
| nonCriticalExtensions SEQUENCE { |  |  |  |
| measIdleConfig-r16 CHOICE { |  |  |  |
| setup | MeasIdleConfigDedicated-r16 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.11.1.3.3-2: MeasIdleConfigDedicated (all steps, Table 8.1.5.11.1.3.3-1)

| Derivation Path: TS 38.331 [12] Chapter 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasIdleConfigDedicated-r16 ::= SEQUENCE { |  |  |  |
| measIdleCarrierListNR-r16 | Not present |  |  |
| measIdleCarrierListEUTRA-r16 | Not present |  |  |
| measIdleDuration-r16 | sec120 |  |  |
| validityAreaList-r16 | Not present |  |  |
| } |  |  |  |

Table 8.1.5.11.1.3.3-3: UEInformationRequest (steps 11, 26, and 41, Table 8.1.5.11.1.3.2-1)

Derivation Path: TS 38.508-1 [4], table 4.6.1-32A with condition IDLE

Table 8.1.5.11.1.3.3-4: UEInformationResponse (steps 12 and 27, Table 8.1.5.11.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| measResultIdleNR-r16 SEQUENCE { |  |  |  |
| measResultServingCell-r16 SEQUENCE { |  |  |  |
| rsrp-Result-r16 | Any allowed value |  |  |
| rsrq-Result-r16 | Any allowed value |  |  |
| resultsSSB-Indexes-r16 SEQUENCE (SIZE (1.. maxNrofIndexesToReport)) OF ResultsPerSSB-IndexIdle-r16 { | 1 entry |  |  |
| ResultsPerSSB-IndexIdle-r16[1] SEQUENCE { |  | entry 1 |  |
| ssb-Index-r16 | Any allowed value |  |  |
| ssb-Results-r16 SEQUENCE { |  |  |  |
| ssb-RSRP-Result-r16 | Any allowed value |  |  |
| ssb-RSRQ-Result-r16 | Any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultsPerCarrierListIdleNR-r16 SEQUENCE (SIZE (1.. maxFreqIdle-r16)) OF MeasResultsPerCarrierIdleNR-r16 { | 1 entry |  |  |
| MeasResultsPerCarrierIdleNR-r16[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r16 | ARFCN value corresponding to NR cell 3 |  |  |
| measResultsPerCellListIdleNR-r16 SEQUENCE (SIZE (1..maxCellMeasIdle-r16)) OF MeasResultsPerCellIdleNR-r16 { | 1 entry |  |  |
| MeasResultsPerCellIdleNR-r16[1] SEQUENCE { |  | entry 1 |  |
| physCellId-r16 | PhysCellId corresponding to NR cell 3 |  |  |
| measIdleResultNR-r16 SEQUENCE { |  |  |  |
| rsrp-Result-r16 | Any allowed value |  |  |
| rsrq-Result-r16 | Any allowed value |  |  |
| resultsSSB-Indexes-r16 SEQUENCE (SIZE (1.. maxNrofIndexesToReport)) OF ResultsPerSSB-IndexIdle-r16 { | 1 entry |  |  |
| ResultsPerSSB-IndexIdle-r16[1] SEQUENCE { |  | entry 1 |  |
| ssb-Index-r16 | Any allowed value |  |  |
| ssb-Results-r16 SEQUENCE { |  |  |  |
| ssb-RSRP-Result-r16 | Any allowed value |  |  |
| ssb-RSRQ-Result-r16 | Any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.11.1.3.3-5: UEInformationResponse (step 42, Table 8.1.5.11.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| measResultIdleNR-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.1.5.11.2	Idle/Inactive measurements / Idle mode / RRCRelease configuration / Measurement of NR cells

8.1.5.11.2.1	Test Purpose (TP)

(1)

with { UE in NR RRC_IDLE state after receiving RRCRelease message including measIdleCarrierListNR with ssb-MeasConfig while being connected to serving cell in which SIB1 includes idleModeMeasurementsNR and UE supporting idleInactiveNR-MeasReport }

ensure that {

when { UE receives UEInformationRequest message with idleModeMeasurementReq after successful measurement procedure and RRC connection establishment }

then { UE reports measurement results in UEInformationResponse message }

}

(2)

with { UE in NR RRC_IDLE state after receiving RRCRelease message including measIdleCarrierListNR without ssb-MeasConfig while being connected to serving cell in which SIB1 includes idleModeMeasurementsNR and SIB11 is configured with measIdleCarrierListNR with ssb-MeasConfig and UE supporting idleInactiveNR-MeasReport }

ensure that {

when { UE receives UEInformationRequest message with idleModeMeasurementReq after successful measurement procedure and RRC connection establishment }

then { UE reports measurement results in UEInformationResponse message }

}

(3)

with { UE in NR RRC_IDLE state after receiving RRCRelease message including measIdleCarrierListNR without ssb-MeasConfig while being connected to serving cell in which SIB1 includes idleModeMeasurementsNR and SIB11 is configured with measIdleCarrierListNR without ssb-MeasConfig and SIB4 is configured with interFreqCarrierFreqList parameters and UE supporting idleInactiveNR-MeasReport }

ensure that {

when { UE receives UEInformationRequest message with idleModeMeasurementReq after successful measurement procedure and RRC connection establishment }

then { UE reports measurement results in UEInformationResponse message }

}

(4)

with { UE in NR RRC_IDLE state after receiving RRCRelease message including measIdleCarrierListNR without ssb-MeasConfig while being connected to serving cell in which SIB1 includes idleModeMeasurementsNR and SIB11 is not configured  and UE supporting idleInactiveNR-MeasReport }

ensure that {

when { UE receives UEInformationRequest message with idleModeMeasurementReq after unsuccessful measurement procedure and RRC connection establishment }

then { UE does not report measurement results in UEInformationResponse message }

}

8.1.5.11.2.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 38.331 clause 5.3.8.3, 5.7.8 and 5.7.10.3. Unless otherwise stated, these are Rel-16 requirements.

[TS 38.331, clause 5.3.8.3]

The UE shall:

1>	if the RRCRelease includes the measIdleConfig:

2>	if T331 is running:

3> stop timer T331;

3>	perform the actions as specified in 5.7.8.3;

2>	if the measIdleConfig is set to setup:

3>	store the received measIdleDuration in VarMeasIdleConfig;

3>	start timer T331 with the value set to measIdleDuration;

3>	if the measIdleConfig contains measIdleCarrierListNR:

4>	store the received measIdleCarrierListNR in VarMeasIdleConfig;

[TS 38.331, clause 5.7.8.1]

This procedure specifies the measurements to be performed and stored by a UE in RRC_IDLE and RRC_INACTIVE when it has an idle/inactive measurement configuration.

[TS 38.331, clause 5.7.8.1a]

The purpose of this procedure is to update the idle/inactive measurement configuration.

The UE initiates this procedure while T331 is running and one of the following conditions is met:

1>	upon selecting a cell when entering RRC_IDLE or RRC-INACTIVE from RRC_CONNECTED or RRC_INACTIVE; or

1>	upon update of system information (SIB4, or SIB11), e.g. due to intra-RAT cell (re)selection;

While in RRC_IDLE or RRC_INACTIVE, and T331 is running, the UE shall:

1>	for each entry in the measIdleCarrierListNR within VarMeasIdleConfig that does not contain an ssb-MeasConfig received from the RRCRelease message:

2>	if there is an entry in measIdleCarrierListNR in measIdleConfigSIB of SIB11 that has the same carrier frequency and subcarrier spacing as the entry in the measIdleCarrierListNR within VarMeasIdleConfig and that contains ssb-MeasConfig:

3>	delete the ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig;

3>	store the SSB measurement configuration from SIB11 into nrofSS-BlocksToAverage, absThreshSS-BlocksConsolidation, smtc, ssb-ToMeasure, deriveSSB-IndexFromCell, and ss-RSSI-Measurement within ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig;

2>	else if there is an entry in interFreqCarrierFreqList of SIB4 with the same carrier frequency and subcarrier spacing as the entry in measIdleCarrierListNR within VarMeasIdleConfig:

3>	delete the ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig;

3>	store the SSB measurement configuration from SIB4 into nrofSS-BlocksToAverage, absThreshSS-BlocksConsolidation, smtc, ssb-ToMeasure, deriveSSB-IndexFromCell, and ss-RSSI-Measurement within ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig;

2>	else:

3>	remove the ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig, if stored;

1>	perform measurements according to 5.7.8.2a.

[TS 38.331, clause 5.7.8.2a]

When performing measurements on NR carriers according to this clause, the UE shall derive the cell quality as specified in 5.5.3.3 and consider the beam quality to be the value of the measurement results of the concerned beam, where each result is averaged as described in TS 38.215 [9].

While in RRC_IDLE or RRC_INACTIVE, and T331 is running, the UE shall:

1>	perform the measurements in accordance with the following:

2>	if the VarMeasIdleConfig includes the measIdleCarrierListNR and the SIB1 contains idleModeMeasurementsNR:

3>	for each entry in measIdleCarrierListNR within VarMeasIdleConfig that contains ssb-MeasConfig:

4>	if UE supports carrier aggregation or NR-DC between serving carrier and the carrier frequency and subcarrier spacing indicated by carrierFreq and ssbSubCarrierSpacing within the corresponding entry:

5>	perform measurements in the carrier frequency and subcarrier spacing indicated by carrierFreq and ssbSubCarrierSpacing within the corresponding entry;

5>	if the reportQuantities is set to rsrq:

6>	consider RSRQ as the cell sorting quantity;

5>	else:

6>	consider RSRP as the cell sorting quantity;

5>	if the measCellListNR is included:

6>	consider cells identified by each entry within the measCellListNR to be applicable for idle/inactive measurement reporting;

5>	else:

6>	consider up to maxCellMeasIdle strongest identified cells, according to the sorting quantity, to be applicable for idle/inactive measurement reporting;

5>	for all cells applicable for idle/inactive measurement reporting, derive cell measurement results for the measurement quantities indicated by reportQuantities;

5>	store the derived cell measurement results as indicated by reportQuantities for cells applicable for idle/inactive measurement reporting within measResultsPerCarrierListIdleNR in the measReportIdleNR in VarMeasIdleReport in decreasing order of the cell sorting quantity, i.e. the best cell is included first, as follows:

6>	if qualityThreshold is configured:

7>	include the measurement results from the cells applicable for idle/inactive measurement reporting whose RSRP/RSRQ measurement results are above the value(s) provided in qualityThreshold;

6>	else:

7>	include the measurement results from all cells applicable for idle/inactive measurement reporting;

5>	if beamMeasConfigIdle is included in the associated entry in measIdleCarrierListNR and if UE supports idleInactiveNR-MeasBeamReport for the FR of the carrier frequency indicated by carrierFreq within the associated entry, for each cell in the measurement results:

6>	derive beam measurements based on SS/PBCH block for each measurement quantity indicated in reportQuantityRS-Indexes, as described in TS 38.215 [9];

6>	if the reportQuantityRS-Indexes is set to rsrq:

7>	consider RSRQ as the beam sorting quantity;

6>	else:

7>	consider RSRP as the beam sorting quantity;

6>	set resultsSSB-Indexes to include up to maxNrofRS-IndexesToReport SS/PBCH block indexes in order of decreasing beam sorting quantity as follows:

7>	include the index associated to the best beam for the sorting quantity and if absThreshSS-BlocksConsolidation is included, the remaining beams whose sorting quantity is above absThreshSS-BlocksConsolidation;

6>	if the includeBeamMeasurements is set to true:

7>	include the beam measurement results as indicated by reportQuantityRS-Indexes;

2>	if, as a result of the procedure in this subclause, the UE performs measurements in one or more carrier frequency indicated by measIdleCarrierListNR or measIdleCarrierListEUTRA:

3>	store the cell measurement results for RSRP and RSRQ for the serving cell within measResultServingCell in the measReportIdleNR in VarMeasIdleReport.

3>	if the VarMeasIdleConfig includes the measIdleCarrierListNR and it contains an entry with carrierFreq set to the value of the serving frequency:

4>	if beamMeasConfigIdle is included in that entry, and if the UE supports idleInactiveNR- MeasBeamReport for the FR of the serving cell:

5>	derive beam measurements based on SS/PBCH block for each measurement quantity indicated in reportQuantityRS-Indexes, as described in TS 38.215 [9];

5>	if the reportQuantityRS-Indexes is set to rsrq:

6>	consider RSRQ as the beam sorting quantity;

5>	else:

6>	consider RSRP as the beam sorting quantity;

5>	set resultsSSB-Indexes to include up to maxNrofRS-IndexesToReport SS/PBCH block indexes in order of decreasing beam sorting quantity as follows:

6>	include the index associated to the best beam for the sorting quantity and if absThreshSS-BlocksConsolidation is included in SIB2 of serving cell, the remaining beams whose sorting quantity is above absThreshSS-BlocksConsolidation;

5>	if the includeBeamMeasurements is set to true:

6>	include the beam measurement results as indicated by reportQuantityRS-Indexes;

NOTE 1:	How the UE performs idle/inactive measurements is up to UE implementation as long as the requirements in TS 38.133 [14] are met for measurement reporting.

NOTE 2:	The UE is not required to perform idle/inactive measurements on a given carrier if the SSB configuration of that carrier provided via dedicated signalling is different from the SSB configuration broadcasted in the serving cell, if any.

NOTE 3:	How the UE prioritizes which frequencies to measure or report (in case it is configured with more frequencies than it can measure or report) is left to UE implementation.

[TS 38.331, clause 5.7.8.3]

The UE shall:

1>	if T331 expires or is stopped:

2>	release the VarMeasIdleConfig.

NOTE:	It is up to UE implementation whether to continue idle/inactive measurements according to SIB11 and SIB4 configurations or according to E-UTRA SIB5 and E-UTRA SIB24 configurations as specified in TS 36.331 [10] upon inter-RAT cell reselection to E-UTRA, after T331 has expired or stopped.

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

1>	if the idleModeMeasurementReq is included in the UEInformationRequest and the UE has stored VarMeasIdleReport that contains measurement information concerning cells other than the PCell:

2>	set the measResultIdleNR in the UEInformationResponse message to the value of measReportIdleNR in the VarMeasIdleReport, if available;

8.1.5.11.2.3	Test Description

8.1.5.11.2.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1, serving cell. System information combination NR-1 defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used. Condition EMR_NR used for SIB1.

-	NR Cell 3, suitable for inter-frequency measurements. System information combination NR-1 defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used.

UE:

-	None.

Preamble:

-	The UE is in 5GS state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3 on NR Cell 1.

8.1.5.11.2.3.2	Test procedure sequence

Table 8.1.5.11.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits RRCRelease message with measIdleConfig including measIdleCarrierListNR with ssb-MeasConfig to the UE. | <-- | NR RRC: RRCRelease | - | - |
| 2 | Wait for 15 s for UE to enter RRC_IDLE mode and to perform measurements. | - | - | - | - |
| 3-10 | Steps 1 to 8 of procedure described in TS 38.508-1 [4] table 4.5.4.2-3: NR RRC_CONNECTED are performed. | - | - | - | - |
| 11 | The SS transmits UEInformationRequest message including idleModeMeasurementReq to the UE. | <-- | NR RRC: UEInformationRequest | - | - |
| 12 | Check: Does the UE transmit UEInformationResponse including measResultIdleNR? | --> | NR RRC: UEInformationResponse | 1 | P |
| 13 | The SS changes system information combination of NR Cell 1 to NR-19 defined in TS 38.508-1 [4] clause 4.4.3.1.2 with condition EMR_NR for SIB1 and EMR_NR_SIB11 for SIB11. | - | - | - | - |
| 14 | The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. | <-- | PDCCH (DCI 1_0): Short Message | - | - |
| 15 | Wait for 2.1* modification period to allow the new system information to take effect. | - | - | - | - |
| 16 | The SS transmits RRCRelease message with measIdleConfig including measIdleCarrierListNR without ssb-MeasConfig to the UE. | <-- | NR RRC: RRCRelease | - | - |
| 17 | Wait for 15 s for UE to enter RRC_IDLE mode and to perform measurements. | - | - | - | - |
| 18-25 | Steps 1 to 8 of procedure described in TS 38.508-1 [4] table 4.5.4.2-3: NR RRC_CONNECTED are performed. | - | - | - | - |
| 26 | The SS transmits UEInformationRequest message including idleModeMeasurementReq to the UE. | <-- | NR RRC: UEInformationRequest | - | - |
| 27 | Check: Does the UE transmit UEInformationResponse including measResultIdleNR? | --> | NR RRC: UEInformationResponse | 2 | P |
| 28 | The SS changes system information combination of NR Cell 1 to NR-18 defined in TS 38.508-1 [4] clause 4.4.3.1.2 with condition EMR_NR for SIB1 and EMR_NR_SIB4 for SIB11. | - | - | - | - |
| 29 | The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. | <-- | PDCCH (DCI 1_0): Short Message | - | - |
| 30 | Wait for 2.1* modification period to allow the new system information to take effect. | - | - | - | - |
| 31 | The SS transmits RRCRelease message with measIdleConfig including measIdleCarrierListNR without ssb-MeasConfig to the UE. | <-- | NR RRC: RRCRelease | - | - |
| 32 | Wait for 15 s for UE to enter RRC_IDLE mode and to perform measurements. | - | - | - | - |
| 33-40 | Steps 1 to 8 of procedure described in TS 38.508-1 [4] table 4.5.4.2-3: NR RRC_CONNECTED are performed. | - | - | - | - |
| 41 | The SS transmits UEInformationRequest message including idleModeMeasurementReq to the UE. | <-- | NR RRC: UEInformationRequest | - | - |
| 42 | Check: Does the UE transmit UEInformationResponse including measResultIdleNR? | --> | NR RRC: UEInformationResponse | 3 | P |
| 43 | The SS changes system information combination of NR Cell 1 to NR-1 defined in TS 38.508-1 [4] clause 4.4.3.1.2 with condition EMR_NR for SIB1. | - | - | - | - |
| 44 | The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. | <-- | PDCCH (DCI 1_0): Short Message | - | - |
| 45 | Wait for 2.1* modification period to allow the new system information to take effect. | - | - | - | - |
| 46 | The SS transmits RRCRelease message with measIdleConfig including measIdleCarrierListNR without ssb-MeasConfig to the UE. | <-- | NR RRC: RRCRelease | - | - |
| 47 | Wait for 15 s for UE to enter RRC_IDLE mode and to perform measurements. | - | - | - | - |
| 48-55 | Steps 1 to 8 of procedure described in TS 38.508-1 [4] table 4.5.4.2-3: NR RRC_CONNECTED are performed. | - | - | - | - |
| 56 | The SS transmits UEInformationRequest message including idleModeMeasurementReq to the UE. | <-- | NR RRC: UEInformationRequest | - | - |
| 57 | Check: Does the UE transmit UEInformationResponse without measResultIdleNR? | --> | NR RRC: UEInformationResponse | 4 | P |

8.1.5.11.2.3.3	Specific message contents

Table 8.1.5.11.2.3.3-1: RRCRelease (steps 1, 16, 31 and 46, Table 8.1.5.11.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCRelease ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |
| nonCriticalExtensions SEQUENCE { |  |  |  |
| nonCriticalExtensions SEQUENCE { |  |  |  |
| measIdleConfig-r16 CHOICE { |  |  |  |
| setup | MeasIdleConfigDedicated-r16 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.11.2.3.3-2: MeasIdleConfigDedicated (step 1, Table 8.1.5.11.2.3.2-1)

| Derivation Path: TS 38.331 [12], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasIdleConfigDedicated-r16 ::= SEQUENCE { |  |  |  |
| measIdleCarrierListNR-r16 SEQUENCE (SIZE (1..maxFreqIdle-r16)) OF MeasIdleCarrierNR-r16 { | 1 entry |  |  |
| MeasIdleCarrierNR-r16[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r16 | ARFCN value of configured NR cell 3 |  |  |
| ssbSubcarrierSpacing-r16 | SubcarrierSpacing | TS 38.508-1 [4] Table 4.6.3-188 |  |
| frequencyBandList | Not present |  |  |
| measCellListNR-r16 | Not present |  |  |
| reportQuantities-r16 | both |  |  |
| qualityThreshold-r16 | Not present |  |  |
| ssb-MeasConfig-r16 SEQUENCE { |  |  |  |
| nrofSS-BlocksToAverage-r16 | 2 |  |  |
| absThreshSS-BlocksConsolidation-r16 SEQUENCE { |  |  |  |
| thresholdRSRP | RSRP-Range | TS 38.508-1 [4] Table 4.6.3-152 |  |
| thresholdRSRQ | Not present |  |  |
| thresholdSINR | Not present |  |  |
| } |  |  |  |
| smtc-r16 | SSB-MTC | TS 38.508-1 [4] Table 4.6.3-185 |  |
| ssb-ToMeasure-r16 | SSB-ToMeasure | TS 38.508-1 [4] Table 4.6.3-187 |  |
| deriveSSB-IndexFromCell-r16 | false |  |  |
|  | true |  | FR1_TDD, FR2_TDD |
| ss-RSSI-Measurement-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdleCarrierListEUTRA-r16 | Not present |  |  |
| measIdleDuration-r16 | sec120 |  |  |
| validityAreaList-r16 | Not present |  |  |
| } |  |  |  |

Table 8.1.5.11.2.3.3-3: MeasIdleConfigDedicated (steps 16, 31 and 46, Table 8.1.5.11.2.3.2-1)

| Derivation Path: TS 38.331 [12], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasIdleConfigDedicated-r16 ::= SEQUENCE { |  |  |  |
| measIdleCarrierListNR-r16 SEQUENCE (SIZE (1..maxFreqIdle-r16)) OF MeasIdleCarrierNR-r16 { | 1 entry |  |  |
| MeasIdleCarrierNR-r16[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r16 | ARFCN value of configured NR cell 3 |  |  |
| ssbSubcarrierSpacing-r16 | SubcarrierSpacing | TS 38.508-1 [4] Table 4.6.3-188 |  |
| frequencyBandList | Not present |  |  |
| measCellListNR-r16 | Not present |  |  |
| reportQuantities-r16 | both |  |  |
| qualityThreshold-r16 | Not present |  |  |
| ssb-MeasConfig-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdleCarrierListEUTRA-r16 | Not present |  |  |
| measIdleDuration-r16 | sec120 |  |  |
| validityAreaList-r16 | Not present |  |  |
| } |  |  |  |

Table 8.1.5.11.2.3.3-4: UEInformationRequest (steps 11, 26, 41 and 56, Table 8.1.5.11.2.3.2-1)

Derivation Path: TS 38.508-1 [4], table 4.6.1-32A with condition IDLE

Table 8.1.5.11.2.3.3-5: UEInformationResponse (steps 12, 27 and 42, Table 8.1.5.11.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| measResultIdleNR-r16 SEQUENCE { |  |  |  |
| measResultServingCell-r16 SEQUENCE { |  |  |  |
| rsrp-Result-r16 | Any allowed value |  |  |
| rsrq-Result-r16 | Any allowed value |  |  |
| resultsSSB-Indexes-r16 SEQUENCE (SIZE (1.. maxNrofIndexesToReport)) OF ResultsPerSSB-IndexIdle-r16 { | 1 entry |  |  |
| ResultsPerSSB-IndexIdle-r16[1] SEQUENCE { |  | entry 1 |  |
| ssb-Index-r16 | Any allowed value |  |  |
| ssb-Results-r16 SEQUENCE { |  |  |  |
| ssb-RSRP-Result-r16 | Any allowed value |  |  |
| ssb-RSRQ-Result-r16 | Any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultsPerCarrierListIdleNR-r16 SEQUENCE (SIZE (1.. maxFreqIdle-r16)) OF MeasResultsPerCarrierIdleNR-r16 { | 1 entry |  |  |
| MeasResultsPerCarrierIdleNR-r16[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r16 | ARFCN value corresponding to NR cell 3 |  |  |
| measResultsPerCellListIdleNR-r16 SEQUENCE (SIZE (1..maxCellMeasIdle-r16)) OF MeasResultsPerCellIdleNR-r16 { | 1 entry |  |  |
| MeasResultsPerCellIdleNR-r16[1] SEQUENCE { |  | entry 1 |  |
| physCellId-r16 | PhysCellId corresponding to NR cell 3 |  |  |
| measIdleResultNR-r16 SEQUENCE { |  |  |  |
| rsrp-Result-r16 | Any allowed value |  |  |
| rsrq-Result-r16 | Any allowed value |  |  |
| resultsSSB-Indexes-r16 SEQUENCE (SIZE (1.. maxNrofIndexesToReport)) OF ResultsPerSSB-IndexIdle-r16 { | 1 entry |  |  |
| ResultsPerSSB-IndexIdle-r16[1] SEQUENCE { |  | entry 1 |  |
| ssb-Index-r16 | Any allowed value |  |  |
| ssb-Results-r16 SEQUENCE { |  |  |  |
| ssb-RSRP-Result-r16 | Any allowed value |  |  |
| ssb-RSRQ-Result-r16 | Any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.11.2.3.3-6: UEInformationResponse (step 57, Table 8.1.5.11.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| measResultIdleNR-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.1.5.11.3	Idle/Inactive measurements / Inactive mode / SIB11 configuration / Measurement of NR cells

8.1.5.11.3.1	Test Purpose (TP)

(1)

with { UE in NR RRC_INACTIVE state after receiving RRCRelease message with suspendConfig and measIdleConfig not including measIdleCarrierListNR while being connected to serving cell in which SIB1 includes idleModeMeasurementsNR and SIB11 is configured with measIdleCarrierListNR with ssb-MeasConfig and UE supporting idleInactiveNR-MeasReport }

ensure that {

when { UE receives UEInformationRequest message with idleModeMeasurementReq after successful measurement procedure and receiving RRCResume message without idleModeMeasurementReq }

then { UE reports measurement results in UEInformationResponse message }

}

(2)

with { UE in NR RRC_INACTIVE state after receiving RRCRelease message with suspendConfig and measIdleConfig not including measIdleCarrierListNR while being connected to serving cell in which SIB1 includes idleModeMeasurementsNR, SIB11 is configured with measIdleCarrierListNR without ssb-MeasConfig and SIB4 is configured with interFreqCarrierFreqList parameters and UE supporting idleInactiveNR-MeasReport }

ensure that {

when { UE receives RRCResume message with idleModeMeasurementReq after successful measurement procedure }

then { UE reports measurement results in RRCResumeComplete message }

}

(3)

with { UE in NR RRC_INACTIVE state after receiving RRCRelease message with suspendConfig and measIdleConfig not including measIdleCarrierListNR while being connected to serving cell in which SIB1 includes idleModeMeasurementsNR and SIB11 is not configured and UE supporting idleInactiveNR-MeasReport }

ensure that {

when { UE receives UEInformationRequest message with idleModeMeasurementReq after unsuccessful measurement procedure and receiving RRCResume message without idleModeMeasurementReq }

then { UE does not report measurement results in UEInformationResponse message }

}

8.1.5.11.3.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 38.331 clause 5.3.8.3, 5.7.8, 5.3.13.4 and 5.7.10.3. Unless otherwise stated, these are Rel-16 requirements.

[TS 38.331, clause 5.3.8.3]

1>	if the RRCRelease includes the measIdleConfig:

2>	if T331 is running:

3> stop timer T331;

3>	perform the actions as specified in 5.7.8.3;

2>	if the measIdleConfig is set to setup:

3>	store the received measIdleDuration in VarMeasIdleConfig;

3>	start timer T331 with the value set to measIdleDuration;

[TS 38.331, clause 5.7.8.1a]

While in RRC_IDLE or RRC_INACTIVE, and T331 is running, the UE shall:

1>	if VarMeasIdleConfig includes neither a measIdleCarrierListEUTRA nor a measIdleCarrierListNR received from the RRCRelease message:

2>	if the UE supports idleInactiveNR-MeasReport:

3>	if SIB11 includes the measIdleConfigSIB and contains measIdleCarrierListNR:

4>	store or replace the measIdleCarrierListNR of measIdleConfigSIB of SIB11 within VarMeasIdleConfig;

3>	else:

4>	remove the measIdleCarrierListNR in VarMeasIdleConfig, if stored;

1>	for each entry in the measIdleCarrierListNR within VarMeasIdleConfig that does not contain an ssb-MeasConfig received from the RRCRelease message:

2>	if there is an entry in measIdleCarrierListNR in measIdleConfigSIB of SIB11 that has the same carrier frequency and subcarrier spacing as the entry in the measIdleCarrierListNR within VarMeasIdleConfig and that contains ssb-MeasConfig:

3>	delete the ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig;

3>	store the SSB measurement configuration from SIB11 into nrofSS-BlocksToAverage, absThreshSS-BlocksConsolidation, smtc, ssb-ToMeasure, deriveSSB-IndexFromCell, and ss-RSSI-Measurement within ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig;

2>	else if there is an entry in interFreqCarrierFreqList of SIB4 with the same carrier frequency and subcarrier spacing as the entry in measIdleCarrierListNR within VarMeasIdleConfig:

3>	delete the ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig;

3>	store the SSB measurement configuration from SIB4 into nrofSS-BlocksToAverage, absThreshSS-BlocksConsolidation, smtc, ssb-ToMeasure, deriveSSB-IndexFromCell, and ss-RSSI-Measurement within ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig;

2>	else:

3>	remove the ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig, if stored;

1>	perform measurements according to 5.7.8.2a.

[TS 38.331, clause 5.7.8.2a]

While in RRC_IDLE or RRC_INACTIVE, and T331 is running, the UE shall:

1>	perform the measurements in accordance with the following:

2>	if the VarMeasIdleConfig includes the measIdleCarrierListNR and the SIB1 contains idleModeMeasurementsNR:

3>	for each entry in measIdleCarrierListNR within VarMeasIdleConfig that contains ssb-MeasConfig:

4>	if UE supports carrier aggregation or NR-DC between serving carrier and the carrier frequency and subcarrier spacing indicated by carrierFreq and ssbSubCarrierSpacing within the corresponding entry:

5>	perform measurements in the carrier frequency and subcarrier spacing indicated by carrierFreq and ssbSubCarrierSpacing within the corresponding entry;

5>	if the reportQuantities is set to rsrq:

6>	consider RSRQ as the cell sorting quantity;

5>	else:

6>	consider RSRP as the cell sorting quantity;

5>	if the measCellListNR is included:

6>	consider cells identified by each entry within the measCellListNR to be applicable for idle/inactive measurement reporting;

5>	else:

6>	consider up to maxCellMeasIdle strongest identified cells, according to the sorting quantity, to be applicable for idle/inactive measurement reporting;

5>	for all cells applicable for idle/inactive measurement reporting, derive cell measurement results for the measurement quantities indicated by reportQuantities;

5>	store the derived cell measurement results as indicated by reportQuantities for cells applicable for idle/inactive measurement reporting within measResultsPerCarrierListIdleNR in the measReportIdleNR in VarMeasIdleReport in decreasing order of the cell sorting quantity, i.e. the best cell is included first, as follows:

6>	if qualityThreshold is configured:

7>	include the measurement results from the cells applicable for idle/inactive measurement reporting whose RSRP/RSRQ measurement results are above the value(s) provided in qualityThreshold;

6>	else:

7>	include the measurement results from all cells applicable for idle/inactive measurement reporting;

2>	if, as a result of the procedure in this subclause, the UE performs measurements in one or more carrier frequency indicated by measIdleCarrierListNR or measIdleCarrierListEUTRA:

3>	store the cell measurement results for RSRP and RSRQ for the serving cell within measResultServingCell in the measReportIdleNR in VarMeasIdleReport.

NOTE 1:	How the UE performs idle/inactive measurements is up to UE implementation as long as the requirements in TS 38.133 [14] are met for measurement reporting.

NOTE 2:	The UE is not required to perform idle/inactive measurements on a given carrier if the SSB configuration of that carrier provided via dedicated signalling is different from the SSB configuration broadcasted in the serving cell, if any.

NOTE 3:	How the UE prioritizes which frequencies to measure or report (in case it is configured with more frequencies than it can measure or report) is left to UE implementation.

[TS 38.331, clause 5.7.8.3]

1>	if T331 expires or is stopped:

2>	release the VarMeasIdleConfig.

NOTE:	It is up to UE implementation whether to continue idle/inactive measurements according to SIB11 and SIB4 configurations or according to E-UTRA SIB5 and E-UTRA SIB24 configurations as specified in TS 36.331 [10] upon inter-RAT cell reselection to E-UTRA, after T331 has expired or stopped.

[TS 38.331, clause 5.3.13.4]

The UE shall:

1>	set the content of the of RRCResumeComplete message as follows:

2>	if the UE has idle/inactive measurement information concerning cells other than the PCell available in VarMeasIdleReport:

3>	if the idleModeMeasurementReq is included in the RRCResume message:

4>	set the measResultIdleNR in the RRCResumeComplete message to the value of measReportIdleNR in the VarMeasIdleReport, if available;

4>	discard the VarMeasIdleReport upon successful delivery of the RRCResumeComplete message is confirmed by lower layers;

3>	else:

4>	if the SIB1 contains idleModeMeasurementsNR and the UE has NR idle/inactive measurement information concerning cells other than the PCell available in VarMeasIdleReport; or

5>	include the idleMeasAvailable;

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

1>	if the idleModeMeasurementReq is included in the UEInformationRequest and the UE has stored VarMeasIdleReport that contains measurement information concerning cells other than the PCell:

2>	set the measResultIdleNR in the UEInformationResponse message to the value of measReportIdleNR in the VarMeasIdleReport, if available;

2>	discard the VarMeasIdleReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

8.1.5.11.3.3	Test Description

8.1.5.11.3.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1, serving cell. System information combination NR-19 defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used. Condition EMR_NR used for SIB1 and condition EMR_NR_SIB11 used for SIB11.

-	NR Cell 3, suitable for inter-frequency measurements. System information combination NR-1 defined in TS 38.508-1 [4] clause 4.4.3.1.2.

UE:

-	None.

Preamble:

-	The UE is in 5GS state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3 on NR Cell 1.

8.1.5.11.3.3.2	Test procedure sequence

Table 8.1.5.11.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits RRCRelease message including suspendConfig and measIdleConfig without measIdleCarrierListNR to the UE. | <-- | NR RRC: RRCRelease | - | - |
| 2 | Wait for 15 s for UE to enter RRC_INACTIVE mode and to perform measurements. | - | - | - | - |
| 3 | SS transmits Paging message to UE. | <-- | NR RRC: Paging | - | - |
| 4 | UE transmits RRCResumeRequest message to SS. | --> | NR RRC: RRCResumeRequest | - | - |
| 5 | SS transmits RRCResume message without idleModeMeasurementReq to UE. | <-- | NR RRC: RRCResume | - | - |
| 6 | UE transmits RRCResumeComplete message with idleMeasAvailable to SS. | --> | NR RRC: RRCResumeComplete | - | - |
| 7 | The SS transmits UEInformationRequest message including idleModeMeasurementReq to the UE. | <-- | NR RRC: UEInformationRequest | - | - |
| 8 | Check: Does the UE transmit UEInformationResponse including measResultIdleNR? | --> | NR RRC: UEInformationResponse | 1 | P |
| 9 | The SS changes system information combination of NR Cell 1 to NR-18 defined in TS 38.508-1 [4] clause 4.4.3.1.2 with condition EMR_NR for SIB1 and EMR_NR_SIB4 for SIB11. | - | - | - | - |
| 10 | The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. | <-- | PDCCH (DCI 1_0): Short Message | - | - |
| 11 | Wait for 2.1* modification period to allow the new system information to take effect. | - | - | - | - |
| 12 | The SS transmits RRCRelease message including suspendConfig and measIdleConfig without measIdleCarrierListNR to the UE. | <-- | NR RRC: RRCRelease | - | - |
| 13 | Wait for 15 s for UE to enter RRC_INACTIVE mode and to perform measurements. | - | - | - | - |
| 14 | SS transmits Paging message to UE. | <-- | NR RRC: Paging | - | - |
| 15 | UE transmits RRCResumeRequest message to SS. | --> | NR RRC: RRCResumeRequest | - | - |
| 16 | SS transmits RRCResume message with idleModeMeasurementReq to UE. | <-- | NR RRC: RRCResume | - | - |
| 17 | Check: Does UE transmit RRCResumeComplete message with measResultIdleNR to SS? | --> | NR RRC: RRCResumeComplete | 2 | P |
| 18 | The SS changes system information combination of NR Cell 1 to NR-1 defined in TS 38.508-1 [4] clause 4.4.3.1.2 with condition EMR_NR for SIB1. | - | - | - | - |
| 19 | The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. | <-- | PDCCH (DCI 1_0): Short Message | - | - |
| 20 | Wait for 2.1* modification period to allow the new system information to take effect. | - | - | - | - |
| 21 | The SS transmits RRCRelease message including suspendConfig and measIdleConfig without measIdleCarrierListNR to the UE. | <-- | NR RRC: RRCRelease | - | - |
| 22 | Wait for 15 s for UE to enter RRC_INACTIVE mode and to perform measurements. | - | - | - | - |
| 23 | SS transmits Paging message to UE. | <-- | NR RRC: Paging | - | - |
| 24 | UE transmits RRCResumeRequest message to SS. | --> | NR RRC: RRCResumeRequest | - | - |
| 25 | SS transmits RRCResume message without idleModeMeasurementReq to UE. | <-- | NR RRC: RRCResume | - | - |
| 26 | UE transmits RRCResumeComplete without idleMeasAvailable message to SS. | --> | NR RRC: RRCResumeComplete | - | - |
| 27 | The SS transmits UEInformationRequest message including idleModeMeasurementReq to the UE. | <-- | NR RRC: UEInformationRequest | - | - |
| 28 | Check: Does the UE transmit UEInformationResponse without measResultIdleNR? | --> | NR RRC: UEInformationResponse | 3 | P |

8.1.5.11.3.3.3	Specific message contents

Table 8.1.5.11.3.3.3-1: RRCRelease (steps 1, 12 and 21, Table 8.1.5.11.3.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-16 with condition NR_RRC_INACTIVE |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |
| nonCriticalExtensions SEQUENCE { |  |  |  |
| nonCriticalExtensions SEQUENCE { |  |  |  |
| measIdleConfig-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| measIdleCarrierListNR-r16 | Not present |  |  |
| measIdleCarrierListEUTRA-r16 | Not present |  |  |
| measIdleDuration-r16 | sec120 |  |  |
| validityAreaList-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.11.3.3.3-2: Void

Table 8.1.5.11.3.3.3-3: RRCResume (step 16, Table 8.1.5.11.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], table 4.6.1-17 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCResume ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcResume SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| idleModeMeasurementReq-r16 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.11.3.3.3-4: RRCResumeComplete (step 6, Table 8.1.5.11.3.3.2-1)

| Derivation Path: TS 38.508 [4], table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCResumeComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcResumeComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| idleMeasAvailable-r16 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.11.3.3.3-5: RRCResumeComplete (step 17, Table 8.1.5.11.3.3.2-1)

| Derivation Path: TS 38.508 [4], table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCResumeComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcResumeComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| measResultIdleNR-r16 | MeasResultIdleNR | Table 8.1.5.11.3.3.3-9 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.11.3.3.3-6: UEInformationRequest (steps 7 and 27, Table 8.1.5.11.3.3.2-1)

Derivation Path: TS 38.508-1 [4], table 4.6.1-32A with condition IDLE

Table 8.1.5.11.3.3.3-7: UEInformationResponse (step 8, Table 8.1.5.11.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| measResultIdleNR-r16 | MeasResultIdleNR | Table 8.1.5.11.3.3.3-9 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.11.3.3.3-8: UEInformationResponse (step 28, Table 8.1.5.11.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| measResultIdleNR-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.11.3.3.3-9: MeasResultIdleNR (Table 8.1.5.11.3.3.3-5 and Table 8.1.5.11.3.3.3-7)

| Derivation Path: TS 38.508-1 [4], table 4.6.3-79D |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResultIdleNR-r16 ::= SEQUENCE { |  |  |  |
| measResultServingCell-r16 SEQUENCE { |  |  |  |
| rsrp-Result-r16 | Any allowed value |  |  |
| rsrq-Result-r16 | Any allowed value |  |  |
| resultsSSB-Indexes-r16 SEQUENCE (SIZE (1.. maxNrofIndexesToReport)) OF ResultsPerSSB-IndexIdle-r16 { | 1 entry |  |  |
| ResultsPerSSB-IndexIdle-r16[1] SEQUENCE { |  | entry 1 |  |
| ssb-Index-r16 | Any allowed value |  |  |
| ssb-Results-r16 SEQUENCE { |  |  |  |
| ssb-RSRP-Result-r16 | Any allowed value |  |  |
| ssb-RSRQ-Result-r16 | Any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultsPerCarrierListIdleNR-r16 SEQUENCE (SIZE (1.. maxFreqIdle-r16)) OF MeasResultsPerCarrierIdleNR-r16 { | 1 entry |  |  |
| MeasResultsPerCarrierIdleNR-r16[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r16 | ARFCN value corresponding to NR cell 3 |  |  |
| measResultsPerCellListIdleNR-r16 SEQUENCE (SIZE (1..maxCellMeasIdle-r16)) OF MeasResultsPerCellIdleNR-r16 { | 1 entry |  |  |
| MeasResultsPerCellIdleNR-r16[1] SEQUENCE { |  | entry 1 |  |
| physCellId-r16 | PhysCellId corresponding to NR cell 3 |  |  |
| measIdleResultNR-r16 SEQUENCE { |  |  |  |
| rsrp-Result-r16 | Any allowed value |  |  |
| rsrq-Result-r16 | Any allowed value |  |  |
| resultsSSB-Indexes-r16 SEQUENCE (SIZE (1.. maxNrofIndexesToReport)) OF ResultsPerSSB-IndexIdle-r16 { | 1 entry |  |  |
| ResultsPerSSB-IndexIdle-r16[1] SEQUENCE { |  | entry 1 |  |
| ssb-Index-r16 | Any allowed value |  |  |
| ssb-Results-r16 SEQUENCE { |  |  |  |
| ssb-RSRP-Result-r16 | Any allowed value |  |  |
| ssb-RSRQ-Result-r16 | Any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.1.5.11.4	Idle/Inactive measurements / Inactive mode / RRCRelease configuration / Measurement of NR cells

8.1.5.11.4.1	Test Purpose (TP)

(1)

with { UE in NR RRC_INACTIVE state after receiving RRCRelease message with suspendConfig and measIdleConfig configured with measIdleCarrierListNR with ssb-MeasConfig while being connected to serving cell in which SIB1 includes idleModeMeasurementsNR and UE supporting idleInactiveNR-MeasReport }

ensure that {

when { UE receives UEInformationRequest message with idleModeMeasurementReq after successful measurement procedure and receiving RRCResume message without idleModeMeasurementReq }

then { UE reports measurement results in UEInformationResponse message }

}

(2)

with { UE in NR RRC_INACTIVE state after receiving RRCRelease message with suspendConfig and measIdleConfig configured with measIdleCarrierListNR without ssb-MeasConfig while being connected to serving cell in which in which SIB1 includes idleModeMeasurementsNR and SIB11 is configured with measIdleCarrierListNR with ssb-MeasConfig and UE supporting idleInactiveNR-MeasReport }

ensure that {

when { UE receives RRCResume message with idleModeMeasurementReq after successful measurement procedure }

then { UE reports measurement results in RRCResumeComplete message }

}

(3)

with { UE in NR RRC_INACTIVE state after receiving RRCRelease message with suspendConfig and measIdleConfig configured with measIdleCarrierListNR without ssb-MeasConfig while being connected to serving cell in which SIB1 includes idleModeMeasurementsNR and SIB11 is configured with measIdleCarrierListNR without ssb-MeasConfig and SIB4 is configured with interFreqCarrierFreqList parameters and UE supporting idleInactiveNR-MeasReport }

ensure that {

when { UE receives RRCResume message with idleModeMeasurementReq after successful measurement procedure }

then { UE reports measurement results in RRCResumeComplete message }

}

(4)

with { UE in NR RRC_INACTIVE state after receiving RRCRelease message with suspendConfig and measIdleConfig including measIdleCarrierListNR without ssb-MeasConfig while being connected to serving cell in which SIB1 includes idleModeMeasurementsNR and UE supporting idleInactiveNR-MeasReport }

ensure that {

when { UE receives UEInformationRequest message with idleModeMeasurementReq after unsuccessful measurement procedure and receiving RRCResume message without idleModeMeasurementReq }

then { UE does not report measurement results in UEInformationResponse message }

}

8.1.5.11.4.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 38.331 clause 5.3.8.3, 5.7.8, 5.3.13.4 and 5.7.10.3. Unless otherwise stated, these are Rel-16 requirements.

[TS 38.331, clause 5.3.8.3]

The UE shall:

1>	if the RRCRelease includes the measIdleConfig:

2>	if T331 is running:

3> stop timer T331;

3>	perform the actions as specified in 5.7.8.3;

2>	if the measIdleConfig is set to setup:

3>	store the received measIdleDuration in VarMeasIdleConfig;

3>	start timer T331 with the value set to measIdleDuration;

3>	if the measIdleConfig contains measIdleCarrierListNR:

4>	store the received measIdleCarrierListNR in VarMeasIdleConfig;

[TS 38.331, clause 5.7.8.1]

This procedure specifies the measurements to be performed and stored by a UE in RRC_IDLE and RRC_INACTIVE when it has an idle/inactive measurement configuration.

[TS 38.331, clause 5.7.8.1a]

The purpose of this procedure is to update the idle/inactive measurement configuration.

The UE initiates this procedure while T331 is running and one of the following conditions is met:

1>	upon selecting a cell when entering RRC_IDLE or RRC-INACTIVE from RRC_CONNECTED or RRC_INACTIVE; or

1>	upon update of system information (SIB4, or SIB11), e.g. due to intra-RAT cell (re)selection;

While in RRC_IDLE or RRC_INACTIVE, and T331 is running, the UE shall:

1>	for each entry in the measIdleCarrierListNR within VarMeasIdleConfig that does not contain an ssb-MeasConfig received from the RRCRelease message:

2>	if there is an entry in measIdleCarrierListNR in measIdleConfigSIB of SIB11 that has the same carrier frequency and subcarrier spacing as the entry in the measIdleCarrierListNR within VarMeasIdleConfig and that contains ssb-MeasConfig:

3>	delete the ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig;

3>	store the SSB measurement configuration from SIB11 into nrofSS-BlocksToAverage, absThreshSS-BlocksConsolidation, smtc, ssb-ToMeasure, deriveSSB-IndexFromCell, and ss-RSSI-Measurement within ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig;

2>	else if there is an entry in interFreqCarrierFreqList of SIB4 with the same carrier frequency and subcarrier spacing as the entry in measIdleCarrierListNR within VarMeasIdleConfig:

3>	delete the ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig;

3>	store the SSB measurement configuration from SIB4 into nrofSS-BlocksToAverage, absThreshSS-BlocksConsolidation, smtc, ssb-ToMeasure, deriveSSB-IndexFromCell, and ss-RSSI-Measurement within ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig;

2>	else:

3>	remove the ssb-MeasConfig of the corresponding entry in the measIdleCarrierListNR within VarMeasIdleConfig, if stored;

1>	perform measurements according to 5.7.8.2a.

[TS 38.331, clause 5.7.8.2a]

When performing measurements on NR carriers according to this clause, the UE shall derive the cell quality as specified in 5.5.3.3 and consider the beam quality to be the value of the measurement results of the concerned beam, where each result is averaged as described in TS 38.215 [9].

While in RRC_IDLE or RRC_INACTIVE, and T331 is running, the UE shall:

1>	perform the measurements in accordance with the following:

2>	if the VarMeasIdleConfig includes the measIdleCarrierListNR and the SIB1 contains idleModeMeasurementsNR:

3>	for each entry in measIdleCarrierListNR within VarMeasIdleConfig that contains ssb-MeasConfig:

4>	if UE supports carrier aggregation or NR-DC between serving carrier and the carrier frequency and subcarrier spacing indicated by carrierFreq and ssbSubCarrierSpacing within the corresponding entry:

5>	perform measurements in the carrier frequency and subcarrier spacing indicated by carrierFreq and ssbSubCarrierSpacing within the corresponding entry;

5>	if the reportQuantities is set to rsrq:

6>	consider RSRQ as the cell sorting quantity;

5>	else:

6>	consider RSRP as the cell sorting quantity;

5>	if the measCellListNR is included:

6>	consider cells identified by each entry within the measCellListNR to be applicable for idle/inactive measurement reporting;

5>	else:

6>	consider up to maxCellMeasIdle strongest identified cells, according to the sorting quantity, to be applicable for idle/inactive measurement reporting;

5>	for all cells applicable for idle/inactive measurement reporting, derive cell measurement results for the measurement quantities indicated by reportQuantities;

5>	store the derived cell measurement results as indicated by reportQuantities for cells applicable for idle/inactive measurement reporting within measResultsPerCarrierListIdleNR in the measReportIdleNR in VarMeasIdleReport in decreasing order of the cell sorting quantity, i.e. the best cell is included first, as follows:

6>	if qualityThreshold is configured:

7>	include the measurement results from the cells applicable for idle/inactive measurement reporting whose RSRP/RSRQ measurement results are above the value(s) provided in qualityThreshold;

6>	else:

7>	include the measurement results from all cells applicable for idle/inactive measurement reporting;

5>	if beamMeasConfigIdle is included in the associated entry in measIdleCarrierListNR and if UE supports idleInactiveNR-MeasBeamReport for the FR of the carrier frequency indicated by carrierFreq within the associated entry, for each cell in the measurement results:

6>	derive beam measurements based on SS/PBCH block for each measurement quantity indicated in reportQuantityRS-Indexes, as described in TS 38.215 [9];

6>	if the reportQuantityRS-Indexes is set to rsrq:

7>	consider RSRQ as the beam sorting quantity;

6>	else:

7>	consider RSRP as the beam sorting quantity;

6>	set resultsSSB-Indexes to include up to maxNrofRS-IndexesToReport SS/PBCH block indexes in order of decreasing beam sorting quantity as follows:

7>	include the index associated to the best beam for the sorting quantity and if absThreshSS-BlocksConsolidation is included, the remaining beams whose sorting quantity is above absThreshSS-BlocksConsolidation;

6>	if the includeBeamMeasurements is set to true:

7>	include the beam measurement results as indicated by reportQuantityRS-Indexes;

2>	if, as a result of the procedure in this subclause, the UE performs measurements in one or more carrier frequency indicated by measIdleCarrierListNR or measIdleCarrierListEUTRA:

3>	store the cell measurement results for RSRP and RSRQ for the serving cell within measResultServingCell in the measReportIdleNR in VarMeasIdleReport.

3>	if the VarMeasIdleConfig includes the measIdleCarrierListNR and it contains an entry with carrierFreq set to the value of the serving frequency:

4>	if beamMeasConfigIdle is included in that entry, and if the UE supports idleInactiveNR- MeasBeamReport for the FR of the serving cell:

5>	derive beam measurements based on SS/PBCH block for each measurement quantity indicated in reportQuantityRS-Indexes, as described in TS 38.215 [9];

5>	if the reportQuantityRS-Indexes is set to rsrq:

6>	consider RSRQ as the beam sorting quantity;

5>	else:

6>	consider RSRP as the beam sorting quantity;

5>	set resultsSSB-Indexes to include up to maxNrofRS-IndexesToReport SS/PBCH block indexes in order of decreasing beam sorting quantity as follows:

6>	include the index associated to the best beam for the sorting quantity and if absThreshSS-BlocksConsolidation is included in SIB2 of serving cell, the remaining beams whose sorting quantity is above absThreshSS-BlocksConsolidation;

5>	if the includeBeamMeasurements is set to true:

6>	include the beam measurement results as indicated by reportQuantityRS-Indexes;

NOTE 1:	How the UE performs idle/inactive measurements is up to UE implementation as long as the requirements in TS 38.133 [14] are met for measurement reporting.

NOTE 2:	The UE is not required to perform idle/inactive measurements on a given carrier if the SSB configuration of that carrier provided via dedicated signalling is different from the SSB configuration broadcasted in the serving cell, if any.

NOTE 3:	How the UE prioritizes which frequencies to measure or report (in case it is configured with more frequencies than it can measure or report) is left to UE implementation.

[TS 38.331, clause 5.7.8.3]

The UE shall:

1>	if T331 expires or is stopped:

2>	release the VarMeasIdleConfig.

NOTE:	It is up to UE implementation whether to continue idle/inactive measurements according to SIB11 and SIB4 configurations or according to E-UTRA SIB5 and E-UTRA SIB24 configurations as specified in TS 36.331 [10] upon inter-RAT cell reselection to E-UTRA, after T331 has expired or stopped.

[TS 38.331, clause 5.3.13.4]

The UE shall:

1>	set the content of the of RRCResumeComplete message as follows:

2>	if the UE has idle/inactive measurement information concerning cells other than the PCell available in VarMeasIdleReport:

3>	if the idleModeMeasurementReq is included in the RRCResume message:

4>	set the measResultIdleEUTRA in the RRCResumeComplete message to the value of measReportIdleEUTRA in the VarMeasIdleReport, if available;

4>	set the measResultIdleNR in the RRCResumeComplete message to the value of measReportIdleNR in the VarMeasIdleReport, if available;

4>	discard the VarMeasIdleReport upon successful delivery of the RRCResumeComplete message is confirmed by lower layers;

3>	else:

4>	if the SIB1 contains idleModeMeasurementsNR and the UE has NR idle/inactive measurement information concerning cells other than the PCell available in VarMeasIdleReport; or

4>	if the SIB1 contains idleModeMeasurementsEUTRA and the UE has E-UTRA idle/inactive measurement information available in VarMeasIdleReport:

5>	include the idleMeasAvailable;

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

1>	if the idleModeMeasurementReq is included in the UEInformationRequest and the UE has stored VarMeasIdleReport that contains measurement information concerning cells other than the PCell:

2>	set the measResultIdleNR in the UEInformationResponse message to the value of measReportIdleNR in the VarMeasIdleReport, if available;

2>	discard the VarMeasIdleReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

8.1.5.11.4.3	Test Description

8.1.5.11.4.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1, serving cell. System information combination NR-1 defined in TS 38.508-1 [4] clause 4.4.3.1.2. Condition EMR_NR used for SIB1.

-	NR Cell 3, suitable for inter-frequency measurements. System information combination NR-1 defined in TS 38.508-1 [4] clause 4.4.3.1.2.

UE:

-	None.

Preamble:

-	The UE is in 5GS state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3 on NR Cell 1.

8.1.5.11.4.3.2	Test procedure sequence

Table 8.1.5.11.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits RRCRelease message with suspendConfig and measIdleConfig including measIdleCarrierListNR with ssb-MeasConfig to the UE. | <-- | NR RRC: RRCRelease | - | - |
| 2 | Wait for 15 s for UE to enter RRC_INACTIVE mode and to perform measurements. | - | - | - | - |
| 3 | SS transmits Paging message to UE. | <-- | NR RRC: Paging | - | - |
| 4 | UE transmits RRCResumeRequest message to SS. | --> | NR RRC: RRCResumeRequest | - | - |
| 5 | SS transmits RRCResume message without idleModeMeasurementReq to UE. | <-- | NR RRC: RRCResume | - | - |
| 6 | UE transmits RRCResumeComplete message with idleMeasAvailable to SS. | --> | NR RRC: RRCResumeComplete | - | - |
| 7 | The SS transmits UEInformationRequest message including idleModeMeasurementReq to the UE. | <-- | NR RRC: UEInformationRequest | - | - |
| 8 | Check: Does the UE transmit UEInformationResponse including measResultIdleNR? | --> | NR RRC: UEInformationResponse | 1 | P |
| 9 | The SS changes system information combination of NR Cell 1 to NR-19 defined in TS 38.508-1 [4] clause 4.4.3.1.2 with condition EMR_NR for SIB1 and EMR_NR_SIB11 for SIB11. | - | - | - | - |
| 10 | The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. | <-- | PDCCH (DCI 1_0): Short Message | - | - |
| 11 | Wait for 2.1* modification period to allow the new system information to take effect. | - | - | - | - |
| 12 | The SS transmits RRCRelease message with suspendConfig and measIdleConfig including measIdleCarrierListNR without ssb-MeasConfig to the UE. | <-- | NR RRC: RRCRelease | - | - |
| 13 | Wait for 15 s for UE to enter RRC_INACTIVE mode and to perform measurements. | - | - | - | - |
| 14 | SS transmits Paging message to UE. | <-- | NR RRC: Paging | - | - |
| 15 | UE transmits RRCResumeRequest message to SS. | --> | NR RRC: RRCResumeRequest | - | - |
| 16 | SS transmits RRCResume message with idleModeMeasurementReq to UE. | <-- | NR RRC: RRCResume | - | - |
| 17 | Check: Does UE transmit RRCResumeComplete message including measResultIdleNR to SS? | --> | NR RRC: RRCResumeComplete | 2 | P |
| 18 | The SS changes system information combination of NR Cell 1 to NR-18 defined in TS 38.508-1 [4] clause 4.4.3.1.2 with condition EMR_NR for SIB1 and EMR_NR_SIB4 for SIB11. | - | - | - | - |
| 19 | The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. | <-- | PDCCH (DCI 1_0): Short Message | - | - |
| 20 | Wait for 2.1* modification period to allow the new system information to take effect. | - | - | - | - |
| 21 | The SS transmits RRCRelease message with suspendConfig and measIdleConfig including measIdleCarrierListNR without ssb-MeasConfig to the UE. | <-- | NR RRC: RRCRelease | - | - |
| 22 | Wait for 15 s for UE to enter RRC_INACTIVE mode and to perform measurements. | - | - | - | - |
| 23 | SS transmits Paging message to UE. | <-- | NR RRC: Paging | - | - |
| 24 | UE transmits RRCResumeRequest message to SS. | --> | NR RRC: RRCResumeRequest | - | - |
| 25 | SS transmits RRCResume message with idleModeMeasurementReq to UE. | <-- | NR RRC: RRCResume | - | - |
| 26 | Check: Does UE transmit RRCResumeComplete message including measResultIdleNR to SS? | --> | NR RRC: RRCResumeComplete | 3 | P |
| 27 | The SS changes system information combination of NR Cell 1 to NR-1 defined in TS 38.508-1 [4] clause 4.4.3.1.2 with condition EMR_NR for SIB1. | - | - | - | - |
| 28 | The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. | <-- | PDCCH (DCI 1_0): Short Message | - | - |
| 29 | Wait for 2.1* modification period to allow the new system information to take effect. | - | - | - | - |
| 30 | The SS transmits RRCRelease message with suspendConfig and measIdleConfig including measIdleCarrierListNR without ssb-MeasConfig to the UE. | <-- | NR RRC: RRCRelease | - | - |
| 31 | Wait for 15 s for UE to enter RRC_INACTIVE mode and to perform measurements. | - | - | - | - |
| 32 | SS transmits Paging message to UE. | <-- | NR RRC: Paging | - | - |
| 33 | UE transmits RRCResumeRequest message to SS. | --> | NR RRC: RRCResumeRequest | - | - |
| 34 | SS transmits RRCResume message without idleModeMeasurementReq to UE. | <-- | NR RRC: RRCResume | - | - |
| 35 | UE transmits RRCResumeComplete message without idleMeasAvailable to SS. | --> | NR RRC: RRCResumeComplete | - | - |
| 36 | The SS transmits UEInformationRequest message including idleModeMeasurementReq to the UE. | <-- | NR RRC: UEInformationRequest | - | - |
| 37 | Check: Does the UE transmit UEInformationResponse without measResultIdleNR? | --> | NR RRC: UEInformationResponse | 4 | P |

8.1.5.11.4.3.3	Specific message contents

Table 8.1.5.11.4.3.3-1: RRCRelease (steps 1, 12, 21 and 30, Table 8.1.5.11.4.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-16 with condition NR_RRC_INACTIVE |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |
| nonCriticalExtensions SEQUENCE { |  |  |  |
| nonCriticalExtensions SEQUENCE { |  |  |  |
| measIdleConfig-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| measIdleCarrierListNR-r16 SEQUENCE (SIZE (1..maxFreqIdle-r16)) OF MeasIdleCarrierNR-r16 { | 1 entry |  |  |
| MeasIdleCarrierNR-r16[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r16 | ARFCN value of configured NR cell 3 |  |  |
| ssbSubcarrierSpacing-r16 | SubcarrierSpacing | TS 38.508-1 [4] Table 4.6.3-188 |  |
| frequencyBandList | Not present |  |  |
| measCellListNR-r16 | Not present |  |  |
| reportQuantities-r16 | both |  |  |
| qualityThreshold-r16 | Not present |  |  |
| ssb-MeasConfig-r16 | Not present |  | steps 12, 21 and 30 |
| ssb-MeasConfig-r16 SEQUENCE { |  |  | step 1 |
| nrofSS-BlocksToAverage-r16 | 2 |  |  |
| absThreshSS-BlocksConsolidation-r16 SEQUENCE { |  |  |  |
| thresholdRSRP | RSRP-Range | TS 38.508-1 [4] Table 4.6.3-152 |  |
| thresholdRSRQ | Not present |  |  |
| thresholdSINR | Not present |  |  |
| } |  |  |  |
| smtc-r16 | SSB-MTC | TS 38.508-1 [4] Table 4.6.3-185 |  |
| ssb-ToMeasure-r16 | SSB-ToMeasure | TS 38.508-1 [4] Table 4.6.3-187 |  |
| deriveSSB-IndexFromCell-r16 | false |  |  |
|  | true |  | FR1_TDD, FR2_TDD |
| ss-RSSI-Measurement-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdleCarrierListEUTRA-r16 | Not present |  |  |
| measIdleDuration-r16 | sec120 |  |  |
| validityAreaList-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.11.4.3.3-2: Void

Table 8.1.5.11.4.3.3-3: Void

Table 8.1.5.11.4.3.3-4: RRCResume (steps 16 and 25, Table 8.1.5.11.4.3.2-1)

| Derivation Path: TS 38.508-1 [4], table 4.6.1-17 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCResume ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcResume SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| idleModeMeasurementReq-r16 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.11.4.3.3-5: RRCResumeComplete (step 6, Table 8.1.5.11.4.3.2-1)

| Derivation Path: TS 38.508-1 [4], table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCResumeComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcResumeComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| idleMeasAvailable-r16 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.11.4.3.3-6: RRCResumeComplete (steps 17 and 26, Table 8.1.5.11.4.3.2-1)

| Derivation Path: TS 38.508-1 [4], table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCResumeComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcResumeComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| measResultIdleNR-r16 | MeasResultIdleNR | Table 8.1.5.11.4.3.3-9 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.11.4.3.3-7: UEInformationResponse (step 8, Table 8.1.5.11.4.3.2-1)

| Derivation Path: TS 38.508-1 [4], table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| measResultIdleNR-r16 | MeasResultIdleNR | Table 8.1.5.11.4.3.3-9 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.11.4.3.3-8: UEInformationResponse (step 37, Table 8.1.5.11.4.3.2-1)

| Derivation Path: TS 38.508-1 [4], table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| measResultIdleNR-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.11.4.3.3-9: MeasResultIdleNR (Tables 8.1.5.11.4.3.3-6 and 8.1.5.11.4.3.3-7)

| Derivation Path: TS 38.508-1 [4], table 4.6.3-79D |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| measResultIdleNR-r16 ::= SEQUENCE { |  |  |  |
| measResultServingCell-r16 SEQUENCE { |  |  |  |
| rsrp-Result-r16 | Any allowed value |  |  |
| rsrq-Result-r16 | Any allowed value |  |  |
| resultsSSB-Indexes-r16 SEQUENCE (SIZE (1.. maxNrofIndexesToReport)) OF ResultsPerSSB-IndexIdle-r16 { | 1 entry |  |  |
| ResultsPerSSB-IndexIdle-r16[1] SEQUENCE { |  | entry 1 |  |
| ssb-Index-r16 | Any allowed value |  |  |
| ssb-Results-r16 SEQUENCE { |  |  |  |
| ssb-RSRP-Result-r16 | Any allowed value |  |  |
| ssb-RSRQ-Result-r16 | Any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultsPerCarrierListIdleNR-r16 SEQUENCE (SIZE (1.. maxFreqIdle-r16)) OF MeasResultsPerCarrierIdleNR-r16 { | 1 entry |  |  |
| MeasResultsPerCarrierIdleNR-r16[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r16 | ARFCN value corresponding to NR cell 3 |  |  |
| measResultsPerCellListIdleNR-r16 SEQUENCE (SIZE (1..maxCellMeasIdle-r16)) OF MeasResultsPerCellIdleNR-r16 { | 1 entry |  |  |
| MeasResultsPerCellIdleNR-r16[1] SEQUENCE { |  | entry 1 |  |
| physCellId-r16 | PhysCellId corresponding to NR cell 3 |  |  |
| measIdleResultNR-r16 SEQUENCE { |  |  |  |
| rsrp-Result-r16 | Any allowed value |  |  |
| rsrq-Result-r16 | Any allowed value |  |  |
| resultsSSB-Indexes-r16 SEQUENCE (SIZE (1.. maxNrofIndexesToReport)) OF ResultsPerSSB-IndexIdle-r16 { | 1 entry |  |  |
| ResultsPerSSB-IndexIdle-r16[1] SEQUENCE { |  | entry 1 |  |
| ssb-Index-r16 | Any allowed value |  |  |
| ssb-Results-r16 SEQUENCE { |  |  |  |
| ssb-RSRP-Result-r16 | Any allowed value |  |  |
| ssb-RSRQ-Result-r16 | Any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.1.5.12	Partial Sounding

##### 8.1.5.12.1	Partial Sounding / RRC_CONNECTED / RRCReconfiguration

8.1.5.12.1.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state }

ensure that {

when {  UE receives an RRCReconfiguration message with partialFreqSounding-r17 configured }

then { UE performs SRS transmission using the resource on UL subframe configured by partialFreqSounding-r17 }

}

8.1.5.12.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.214, clause 6.2.1. Unless otherwise stated these are Rel-17 requirements.

[TS 38.214 clause 6.2.1]

The following SRS parameters are semi-statically configurable by higher layer parameter SRS-Resource or SRS-PosResource.

-	srs-ResourceId or SRS-PosResourceId determines SRS resource configuration identity.

-	Number of SRS ports, as defined by the higher layer parameter nrofSRS-Ports and described in clause 6.4.1.4 of [4, TS 38.211]. If not configured, nrofSRS-Ports is 1.

-	Time domain behaviour of SRS resource configuration as indicated by the higher layer parameter resourceType, which may be periodic, semi-persistent, aperiodic SRS transmission as defined in clause 6.4.1.4 of [4, TS 38.211].

-	Slot level periodicity and slot level offset as defined by the higher layer parameters periodicityAndOffset-p or periodicityAndOffset-sp for an SRS resource of type periodic or semi-persistent. The UE is not expected to be configured with SRS resources in the same SRS resource set SRS-ResourceSet or SRS-PosResourceSet with different slot level periodicities. For an SRS-ResourceSet configured with higher layer parameter resourceType set to 'aperiodic', a slot level offset is defined by the higher layer parameter slotOffset. For an SRS-ResourceSet configured with higher layer parameter resourceType set to 'aperiodic', a list of up to four different available slot offset values from the reference slot n + k to the slot where the aperiodic SRS resource set is transmitted where n is the slot with triggering DCI and k is slotOffset, can be configured by the higher layer parameter availableSlotOffsetList. The parameter availableSlotOffsetList can be configured up to 4 different values. For an SRS-PosResourceSet configured with higher layer parameter resourceType set to 'aperiodic', the slot level offset is defined by the higher layer parameter slotOffset for each SRS resource.

-	Number of OFDM symbols in the SRS resource, starting OFDM symbol of the SRS resource within a slot including repetition factor R as defined by the higher layer parameter resourceMapping and described in clause 6.4.1.4 of [4, TS 38.211]. If R is not configured, then R is equal to the number of OFDM symbols in the SRS resource.

-	SRS bandwidth and , as defined by the higher layer parameter freqHopping and described in clause 6.4.1.4 of [4, TS 38.211]. If not configured, then= 0.

-	Frequency hopping bandwidth , as defined by the higher layer parameter freqHopping and described in clause 6.4.1.4 of [4, TS 38.211]. If not configured, then = 0.

-	Defining partial frequency sounding factor and start RB index for partial frequency sounding as defined by the higher layer parameters FreqScalingFactor PF and StartRBIndex kF, respectively, and described in Clause 6.4.1.4 of [4, TS 38.211]. If not configured, then PF = 1 and kF,= 0.

-	Defining start RB index hopping for partial frequency sounding in different SRS frequency hopping periods for aperiodic/periodic/semi-persistent SRS based on the hopping pattern khop as described in clause 6.4.1.4.3 in [4, TS 38.211. If not configured, then start RB hopping is not enabled and khop is fixed to be 0 for all SRS symbols.

[TS 38.211 clause 6.4.1.4.3]

When SRS is transmitted on a given SRS resource, the sequence rpi(n,l') for each OFDM symbol l' and for each of the antenna ports of the SRS resource shall be multiplied with the amplitude scaling factor  in order to conform to the transmit power specified in [5, 38.213] and mapped in sequence starting with  to resource elements  in a slot for each of the antenna ports  according to

The length of the sounding reference signal sequence is given by

Msc,bSRS=mSRS,bNscRBKTCPF

where mSRS,b is given by a selected row of Table 6.4.1.4.3-1 with  where  is given by the field b-SRS contained in the higher-layer parameter freqHopping if configured, otherwise BSRS=0. The row of the table is selected according to the index  given by the field c-SRS contained in the higher-layer parameter freqHopping. The quantity PF ∈2, 4 is given by the higher-layer parameter FreqScalingFactor if configured, otherwise PF=1. When FreqScalingFactor is configured, the UE expects the length of the SRS sequence to be a multiple of 6.

The frequency-domain starting position k0(pi) is defined by

k0(pi)=k0(pi)+noffsetFH+noffsetRPFS

where

k0(pi)=nshiftNscRB+kTC(pi)+koffsetl' mod KTC
kTCpi=kTC+KTC2 mod KTCif NapSRS=4, pi∈1001, 1003, and nSRScs,max=6 kTC+KTC2 mod KTC if NapSRS=4, pi∈1001, 1003, and nSRScs∈nSRScs,max2, …, nSRScs,max-1kTCotherwise
noffsetFH=b=0BSRSmSRS,bNscRBnb
noffsetRPFS=NscRBmSRS,BSRSkF+khopmod PFPF

and

-	kF∈0,1,…,PF-1 is given by the higher-layer parameter StartRBIndex if configured, otherwise kF=0;

-	khop is given by Table 6.4.1.4.3-3 with

khop=nSRSb'=bhopBSRSNb' mod PF
Nbhop=1

if the higher-layer parameter EnableStartRBHopping is configured, otherwise khop=0.

8.1.5.12.1.3	Test description

8.1.5.12.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used in NR cell.

UE:

-	None.

Preamble:

-	The UE is in 5GS state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

8.1.5.12.1.3.2	Test procedure sequence

Table 8.1.5.12.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message to configure the partial sounding with RB location hopping enabled. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | Check: Does the UE transmit SRS on the last symbol every 2 slot, and SRS bandwidth of each transmission is 2RB, and the frequency location of each SRS transmission complies with the configured hopping pattern? NOTE: the verdict is assigned when two full periodicity of SRS transmissions are observed, i.e. 6 SRS transmission if enableStartRBHopping-r17 is ‘Not Present’, or 12 SRS transmissions if enableStartRBHopping-r17 is ‘Enable’. | --> | SRS | 1 | P |

8.1.5.12.1.3.3	Specific message contents

Table 8.1.5.12.1.3.3-1: RRCReconfiguration (step 1, Table 8.1.5.12.1.3.2-1)

| Derivation path: TS 38.508-1 [4] table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions ::= CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| nonCriticalExtension::= SEQUENCE { |  |  |  |
| masterCellGroup::= SEQUENCE { | CellGroupConfig | Table 8.1.5.12.1.3.3-2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.12.1.3.3-2: CellGroupConfig (Table 8.1.5.12.1.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| mac-CellGroupConfig | Not present |  |  |
| physicalCellGroupConfig | Not present |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| rlf-TimersAndConstants | Not present |  |  |
| spCellConfigDedicated | ServingCellConfig | Table 8.1.5.12.1.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.12.1.3.3-3: ServingCellConfig (Table 8.1.5.12.1.3.3-2: ServingCellConfig)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkDedicated | Table 8.1.5.12.1.3.3-4 |  |
| uplinkBWP-ToReleaseList | Not present |  |  |
| uplinkBWP-ToAddModList | Not present |  |  |
| firstActiveUplinkBWP-Id | BWP-Id |  |  |
| pusch-ServingCellConfig | Not present |  |  |
| carrierSwitching | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.12.1.3.3-4: BWP-UplinkDedicated (Table 8.1.5.12.1.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicated ::= SEQUENCE { |  |  |  |
| pucch-Config | Not present |  |  |
| pusch-Config | Not present |  |  |
| srs-Config | SRS-Config | Table 8.1.5.12.1.3.3-5 |  |
| } |  |  |  |

Table 8.1.5.12.1.3.3-5: SRS-Config (Table 8.1.5.12.1.3.3-4)

| Derivation path: TS 38.508-1 [4] table 4.6.3-182 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SRS-Config ::= SEQUENCE { |  |  |  |
| srs-ResourceSetToAddModList SEQUENCE (SIZE(0..maxNrofSRS-ResourceSets)) OF SRS-ResourceSet { | 1 entry |  |  |
| SRS-ResourceSet[1] SEQUENCE { |  | entry 1 |  |
| resourceType CHOICE { |  |  |  |
| periodic SEQUENCE { |  |  |  |
| associatedCSI-RS | NZP-CSI-RS-ResourceId |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| srs-ResourceToAddModList SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SRS-Resource { | 1 entry |  |  |
| SRS-Resource[1] SEQUENCE { |  | entry 1 |  |
| freqHopping SEQUENCE { |  |  |  |
| c-SRS | 2 |  |  |
| b-SRS | 1 |  |  |
| b-hop | 0 |  |  |
| } |  |  |  |
| resourceType CHOICE { |  |  |  |
| periodic SEQUENCE { |  |  |  |
| periodicityAndOffset-p CHOICE { |  |  |  |
| sl2 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| partialFreqSounding-r17 SEQUENCE { |  |  |  |
| startRBIndexFScaling-r17 CHOICE { |  |  |  |
| startRBIndexAndFreqScalingFactor2-r17 | 1 |  |  |
| } |  |  |  |
| enableStartRBHopping-r17 | Not present |  |  |
|  | Enable |  | pc_srs_startRB_locationHoppingPartial_r17 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.1.5.13	Small Data Transmission

##### 8.1.5.13.1	RRC SDT/CG based SDT/Success

8.1.5.13.1.1	Test Purpose (TP)

(1)

with { UE in NR RRC_INACTIVE state and SDT-CG-Config-r17 is configured }

ensure that {

when {  UE has small data to transmit and the data volume of the pending UL data across all RBs configured for SDT is less than or equal to sdt-DataVolumeThreshold and RSRP is above the configured sdt-RSRP-Threshold and at least one SSB configured for CG-SDT with SS-RSRP above cg-SDT-RSRP-ThresholdSSB }

then { UE shall initiate CG based SDT procedure and starts RRC Resume procedure and UE is successfully able to send and receive SDT data}

}

(2)

with { UE in NR RRC_INACTIVE state and SDT-CG-Config-r17 is configured }

ensure that {

when { UE has small data to transmit and the data volume of the pending UL data across all RBs configured for SDT is greater than sdt-DataVolumeThreshold }

then { UE shall not initiate CG based SDT procedure and starts normal RRC Resume procedure }

(3)

with { UE in NR RRC_INACTIVE state and SDT-CG-Config-r17 is configured }

ensure that {

when { UE has small data to transmit and the data volume of the pending UL data across all RBs configured for SDT is less than or equal to sdt-DataVolumeThreshold and RSRP is above the configured sdt-RSRP-Threshold and none of SSB configured for CG-SDT with SS-RSRP above cg-SDT-RSRP-ThresholdSSB are available }

then { UE shall not initiate CG based SDT procedure and starts normal RRC Resume procedure }

8.1.5.13.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331, clauses 5.3.1.1, 5.3.8.3, 5.3.13.1, 5.3.13.1b, 5.3.13.2 and 5.3.13.3. Unless otherwise stated these are Rel-17 requirements.

[TS 38.331 clause 5.3.1.1]

Upon initiating the resume procedure for SDT, AS security (both ciphering and integrity protection) is re-activated for SRB2 (if configured for SDT) and for SRB1. In addition, AS security is also re-activated (if security is configured) for all the DRBs configured for SDT. Further, the PDCP entities of SRB1 and PDCP entities of the radio bearers configured for SDT are re-established and resumed whilst the UE remains in RRC_INACTIVE state. Transmission and reception of data and/or signalling messages over radio bearers configured for SDT can happen whilst the UE is in RRC_INACTIVE state and SDT procedure is ongoing.

In response to a request to resume the RRC connection or in response to a resume procedure initiated for SDT, the network may resume the suspended RRC connection and send UE to RRC_CONNECTED, or reject the request to resume and send UE to RRC_INACTIVE (with a wait timer), or directly re-suspend the RRC connection and send UE to RRC_INACTIVE, or directly release the RRC connection and send UE to RRC_IDLE, or instruct the UE to initiate NAS level recovery (in this case the network sends an RRC setup message).

[TS 38.331 clause 5.3.8.3]

1>	if the RRCRelease includes suspendConfig:

2>	reset MAC and release the default MAC Cell Group configuration, if any;

2>	apply the received suspendConfig except the received nextHopChainingCount;

2>	if the sdt-Config is configured:

3>	for each of the DRB in the sdt-DRB-List:

4>	consider the DRB to be configured for SDT;

3>	if sdt-SRB2-Indication is configured:

4>	consider the SRB2 to be configured for SDT;

3>	for each RLC bearer that is not suspended:

4>	re-establish the RLC entity as specified in TS 38.322 [4];

3>	for SRB2 (if it is resumed) and for SRB1:

4>	trigger the PDCP entity to perform SDU discard as specified in TS 38.323 [5];

3>	if sdt-MAC-PHY-CG-Config is configured:

4>	configure the PCell with the configured grant resources for SDT and instruct the MAC entity to start the cg-SDT-TimeAlignmentTimer;

…

2>	re-establish RLC entities for SRB1;

2>	if the RRCRelease message with suspendConfig was received in response to an RRCResumeRequest or an RRCResumeRequest1:

3>	stop the timer T319 if running;

3>	in the stored UE Inactive AS context:

4>	replace the KgNB and KRRCint keys with the current KgNB and KRRCint keys;

4>	replace the nextHopChainingCount with the value of nextHopChainingCount received in the RRCRelease message;

4>	replace the cellIdentity with the cellIdentity of the cell the UE has received the RRCRelease message;

4>	if the suspendConfig contains the sl-UEIdentityRemote (i.e. the UE is a L2 U2N Remote UE):

…

4> else:

5>	replace the C-RNTI with the C-RNTI used in the cell (see TS 38.321 [3]) the UE has received the RRCRelease message;

5>	replace the physical cell identity with the physical cell identity of the cell the UE has received the RRCRelease message;

3>	replace the nextHopChainingCount with the value associated with the current KgNB;

3>	stop the timer T319a if running and consider SDT procedure is not ongoing;

2>	else:

3>	store in the UE Inactive AS Context the nextHopChainingCount received in the RRCRelease message, the current KgNB and KRRCint keys, the ROHC state, the EHC context(s), the UDC state, the stored QoS flow to DRB mapping rules, the application layer measurement configuration, the C-RNTI used in the source PCell, the cellIdentity and the physical cell identity of the source PCell, the spCellConfigCommon within ReconfigurationWithSync of the NR PSCell (if configured) and all other parameters configured except for:

-	parameters within ReconfigurationWithSync of the PCell;

-	parameters within ReconfigurationWithSync of the NR PSCell, if configured;

-	parameters within MobilityControlInfoSCG of the E-UTRA PSCell, if configured;

-	servingCellConfigCommonSIB;

..

3>	store any previously or subsequently received application layer measurement reports for which no segment, or full message, has been submitted to lower layers for transmission;

NOTE 2:	NR sidelink communication/discovery related configurations and logged measurement configuration are not stored as UE Inactive AS Context, when UE enters RRC_INACTIVE.

2>	suspend all SRB(s) and DRB(s) and multicast MRB(s), except SRB0 and broadcast MRBs;

2>	indicate PDCP suspend to lower layers of all DRBs and multicast MRBs;

2>	release the SRAP entity, if configured;

…

2>	indicate the suspension of the RRC connection to upper layers;

2>	enter RRC_INACTIVE and perform cell selection as specified in TS 38.304 [20];

…

[TS 38.331 clause 5.3.13.1]

The purpose of this procedure is to resume a suspended RRC connection, including resuming SRB(s), DRB(s) and multicast MRB(s) or perform an RNA update. This procedure is also used to initiate SDT in RRC_INACTIVE.

[TS 38.331 clause 5.3.13.1b]

A UE in RRC_INACTIVE initiates the resume procedure for SDT when all of the following conditions are fulfilled:

1>	the upper layers request resumption of RRC connection; and

1>	SIB1 includes sdt-ConfigCommon; and

1>	sdt-Config is configured; and

1>	all the pending data in UL is mapped to the radio bearers configured for SDT; and

1>	lower layers indicate that conditions for initiating SDT as specified in TS 38.321 [3] are fulfilled.

NOTE:	How the UE determines that all pending data in UL is mapped to radio bearers configured for SDT is left to UE implementation.

[TS 38.331 clause 5.3.13.2]

The UE initiates the procedure when upper layers or AS (when responding to RAN paging, upon triggering RNA updates while the UE is in RRC_INACTIVE, for NR sidelink communication/discovery/V2X sidelink communication as specified in clause 5.3.13.1a) requests the resume of a suspended RRC connection or requests the resume for initiating SDT as specified in clause 5.3.13.1b.

…

1>	if sdt-MAC-PHY-CG-Config is configured:

2>	if the resume procedure is initiated in a cell that is different to the PCell in which the UE received the stored sdt-MAC-PHY-CG-Config:

3>	release the stored sdt-MAC-PHY-CG-Config;

3>	instruct the MAC entity to stop the cg-SDT-TimeAlignmentTimer, if it is running;

1>	if conditions for initiating SDT in accordance with 5.3.13.1b are fulfilled:

2>	consider the resume procedure is initiated for SDT;

2>	start timer T319a when the lower layers first transmit the CCCH message;

2>	consider SDT procedure is ongoing;

1> else:

2>	start timer T319;

2>	instruct the MAC entity to stop the cg-SDT-TimeAlignmentTimer, if it is running;

…

[TS 38.331 clause 5.3.13.3]

1>	if the resume procedure is initiated for SDT:

2>	for each radio bearer that is configured for SDT and for SRB1:

3>	restore the RLC-BearerConfig associated with the RLC bearers of masterCellGroup and pdcp-Config from the UE Inactive AS context;

…

3>	re-establish PDCP entity for the radio bearer that is configured for SDT without triggering PDCP status report;

2>	resume all the radio bearers that are configured for SDT;

- submit the selected message RRCResumeRequest or RRCResumeRequest1 for transmission to lower layers.

8.1.5.13.1.3	Test description

8.1.5.13.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used in NR cell.

UE:

-	None.

Preamble:

-	The UE is in 5GS state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3 and Test Loop Function (On) with UE test loop mode B is established.

8.1.5.13.1.3.2	Test procedure sequence

Table 8.1.5.13.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS sends one IP Packet to the UE. The IP Packet is less than sdt-DataVolumeThreshold. (Note 1) | - | - | - | - |
| 2 | The SS transmits an RRCRelease message including SDT-CG-Config-r17 in suspendConfig. | <-- | NR RRC: RRCRelease | - | - |
| 3 | Check: Does the UE transmit an RRCResumeRequest message and the loop backed IP Packet received in step 1 in a CG PUSCH occasion? | --> | NR RRC: RRCResumeRequest | 1 | P |
| 4 | The SS sends one IP Packet to the UE. The IP Packet is less than sdt-DataVolumeThreshold. (Note 1) | - | - | - | - |
| 5 | Check: Does the UE loop back the IP packet received in step 4 in a CG PUSCH occasion? | - | - | 1 | P |
| 6 | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| 7-14 | The test steps 1 to 8 of generic test procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are performed on NR Cell 1. | - | - | - | - |
| 15 | The SS transmits an OPEN UE TEST LOOP message. | <-- | NR RRC: DLInformationTransfer TC: OPEN UE TEST LOOP | - | - |
| 16 | The UE transmits an OPEN UE TEST LOOP COMPLETE message. | --> | NR RRC: ULInformationTransfer TC: OPEN UE TEST LOOP COMPLETE | - | - |
| 17 | The SS transmits a CLOSE UE TEST LOOP message. | <-- | NR RRC: DLInformationTransfer TC: CLOSE UE TEST LOOP | - | - |
| 18 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message. | --> | NR RRC: ULInformationTransfer TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| 19 | The SS sends one IP Packet to the UE. The IP Packet is greater than sdt-DataVolumeThreshold. (Note 2) | - | - | - | - |
| 20 | The SS transmits an RRCRelease message including SDT-CG-Config-r17 in suspendConfig. | <-- | NR RRC: RRCRelease | - | - |
| 21 | Check: Does the UE transmit an RRCResumeRequest message based on random access? | --> | NR RRC: RRCResumeRequest | 2 | P |
| 22 | The SS transmits an RRCResume message. | <-- | NR RRC: RRCResume | - | - |
| 23 | The UE transmits an RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 24 | Check: Does the UE loop back the IP packet received in step 19? | - | - | 2 | P |
| 25 | The SS transmits an OPEN UE TEST LOOP message. | <-- | NR RRC: DLInformationTransfer TC: OPEN UE TEST LOOP | - | - |
| 26 | The UE transmits an OPEN UE TEST LOOP COMPLETE message. | --> | NR RRC: ULInformationTransfer TC: OPEN UE TEST LOOP COMPLETE | - | - |
| 27 | The SS transmits a CLOSE UE TEST LOOP message. | <-- | NR RRC: DLInformationTransfer TC: CLOSE UE TEST LOOP | - | - |
| 28 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message. | --> | NR RRC: ULInformationTransfer TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| 29 | The SS sends one IP Packet to the UE. The IP Packet is less than sdt-DataVolumeThreshold. (Note 1) | - | - | - | - |
| 30 | The SS transmits an RRCRelease message including SDT-CG-Config-r17 in suspendConfig. | <-- | NR RRC: RRCRelease | - | - |
| 31 | Check: Does the UE transmit an RRCResumeRequest message based on random access? | --> | NR RRC: RRCResumeRequest | 3 | P |
| 32 | The SS transmits an RRCResume message. | <-- | NR RRC: RRCResume | - | - |
| 33 | The UE transmits an RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 34 | Check: Does the UE loop back the IP packet received in step 27? | - | - | 3 | P |
| Note 1: RLC PDU for IP packet is 97 bytes (RLC SDU is 94 bytes for 3 bytes RLC header and 95 bytes for 2 bytes RLC header) and sdt-DataVolumeThreshold is 100 bytes. Therefore the size of RLC SDU is less than the sdt-DataVolumeThreshold. Note 2: RLC PDU for IP packet is 104 bytes (RLC SDU is 101 bytes for 3 bytes RLC header and 102 bytes for 2 bytes RLC header) and sdt-DataVolumeThreshold is 100 bytes. Therefore the size of RLC SDU is greater than the sdt-DataVolumeThreshold. |  |  |  |  |  |

8.1.5.13.1.3.3	Specific message contents

Table 8.1.5.13.1.3.3-1: CLOSE UE TEST LOOP (Preamble and all steps, Table 8.1.5.13.1.3.2-1)

| Derivation path: 36.508-1 [7] table 4.7A-3 condition UE test loop mode B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE test loop mode B LB setup |  |  |  |
| IP PDU delay | '0000 0010'B | 2 seconds |  |

Table 8.1.5.13.1.3.3-2: SIB1 (Preamble and all steps, Table 8.1.5.13.1.3.2-1)

| Derivation Path: TS 38.508-1 [4] table 4.6.1-28 with condition SDT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sdt-ConfigCommon-r17 SEQUENCE { |  |  |  |
| sdt-RSRP-Threshold-r17 | 60 | (IE value – 156) dBm = - 96 dBm |  |
| sdt-LogicalChannelSR-DelayTimer-r17 | Not present |  |  |
| sdt-DataVolumeThreshold-r17 | byte100 |  |  |
| t319a-r17 | ms4000 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.13.1.3.3-3: RRCRelease (steps 2, 20 and 30, Table 8.1.5.13.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-16 with condition NR_RRC_INACTIVE and SDT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCRelease ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |
| suspendConfig SEQUENCE { |  |  |  |
| sdt-Config-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sdt-DRB-List-r17 SEQUENCE (SIZE (0..maxDRB)) OF DRB-Identity { | 1 entry |  |  |
| DRB-Identity[1] | DRB-Identity using condition DRBj | Entry 1 j is the ID of the DRB established during the preamble which is allocated according to internal TTCN mapping |  |
| } |  |  |  |
| sdt-MAC-PHY-CG-Config-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cg-SDT-ConfigInitialBWP-NUL-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| pusch-Config-r17 CHOICE { |  |  |  |
| setup | PUSCH-Config | TS 38.508-1[4] Table 4.6.3-118: PUSCH-Config |  |
| } |  |  |  |
| configuredGrantConfigToAddModList-r17 SEQUENCE (SIZE (1..maxNrofConfiguredGrantConfig-r16)) OF ConfiguredGrantConfig { |  |  |  |
| ConfiguredGrantConfig[1] | ConfiguredGrantConfig | Table 8.1.5.13.1.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cg-SDT-ConfigInitialBWP-SUL-r17 | Not present |  |  |
| cg-SDT-ConfigInitialBWP-DL-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| pdcch-Config-r17 | Not present |  |  |
| pdsch-Config-r17 CHOICE { |  |  |  |
| setup | PDSCH-Config | TS 38.508-1[4] Table 4.6.3-100: PDSCH-Config |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cg-SDT-TimeAlignmentTimer-r17 | ms5120 | TimeAlignmentTimer(5.12s) is larger than IP PDU delay timer(2s) for Loop mode B. |  |
| cg-SDT-RSRP-ThresholdSSB-r17 | 60 | (IE value – 156) dBm = - 96 dBm | Step 2,20 |
|  | 76 | (IE value – 156) dBm = - 80 dBm | Step 30 |
| cg-SDT-TA-ValiditationConfig-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cg-SDT-RSRP-ChangeThreshold-r17 | dB10 |  |  |
| } |  |  |  |
| } |  |  |  |
| cg-SDT-CS-RNTI-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.13.1.3.3-4: ConfiguredGrantConfig (Table 8.1.5.13.1.3.3-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-26, condition CG_Config_Type1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ConfiguredGrantConfig ::= SEQUENCE { |  |  |  |
| rrc-ConfiguredUplinkGrant SEQUENCE{ |  |  |  |
| frequencyDomainAllocation | BIT STRING (SIZE(18) | Equals to NBWPsize * (NBWPsize – LRB + 1) + (NBWPsize -1 - RBstart), where LRB = 24 PRB, RBstart = 0, NBWPsize is the size [PRBs] of the carrier bandwidth and contained in TS.38.508-1 [4] clause 6.2.3.1 | SCS15 AND BW5, SCS30 AND BW10 |
|  | BIT STRING (SIZE(18) | Equal to NBWPsize * (LRB-1) + RBstart), where LRB = 15 PRB, RBstart = 0, NBWPsize is the size [PRBs] of the carrier bandwidth and contained in TS.38.508-1 [4] clause 6.2.3.1 | BW3 |
|  | BIT STRING (SIZE(18) | Equal to NBWPsize * (LRB-1) + RBstart), where LRB = 24 PRB, RBstart = 0, NBWPsize is the size [PRBs] of the carrier bandwidth and contained in TS.38.508-1 [4] clause 6.2.3.1 |  |
| dmrs-SeqInitialization | 0 |  |  |
| mcsAndTBS | 0 |  |  |
|  | 2 |  | BW3 |
| } |  |  |  |
| } |  |  |  |
| Note: Configured UL grant for CG-SDT is 888 bits (LRBs & IMCS as per 38.523-3[3] annex B) is chosen to enable UE to transmit RRCResumeRequest and Loop backed RLC PDU in a MAC PDU ( 97 bytes for Loop backed RLC PDU + 2 bytes for MAC Sub Header + 6 bytes for RRCResumeRequest message + 2 bytes for MAC Sub Header + 4 bytes for BSR and Padding ) |  |  |  |

Table 8.1.5.13.1.3.3-5: RRCResumeRequest (steps 3, 21 and 31, Table 8.1.5.13.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCResumeRequest ::= SEQUENCE { |  |  |  |
| rrcResumeRequest SEQUENCE { |  |  |  |
| resumeCause | mo-data |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.1.5.13.2	RRC SDT / CG based SDT ongoing / Data and Signalling on non-SDT Radio Bearers

8.1.5.13.2.1	Test Purpose (TP)

(1)

with { UE in NR RRC_INACTIVE state and CG based SDT procedure is ongoing }

ensure that {

when { UE has data and signalling to transmit on non-SDT Radio Bearers }

then { UE shall send UEAssistanceInformation message with nonSDT-DataIndication set  }

}

8.1.5.13.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 38.331, clause 5.3.1.1, 5.3.8.3, 5.3.13.1, 5.3.13.1b, 5.3.13.2, 5.3.13.3, 5.7.4.2, 5.7.4.3. Unless otherwise stated, these are Rel-17 requirements.

[TS 38.331, clause 5.3.1.1]

The suspension of the RRC connection is initiated by the network. When the RRC connection is suspended, the UE stores the UE Inactive AS context and any configuration received from the network, and transits to RRC_INACTIVE state. The RRC message to suspend the RRC connection is integrity protected and ciphered.

The resumption of a suspended RRC connection is initiated by upper layers when the UE needs to transit from RRC_INACTIVE state to RRC_CONNECTED state or by RRC layer to perform a RNA update or by RAN paging from NG-RAN or for SDT. When the RRC connection is resumed, network configures the UE according to the RRC connection resume procedure based on the stored UE Inactive AS context and any RRC configuration received from the network. The RRC connection resume procedure re-activates AS security and re-establishes SRB(s) and DRB(s) and/or multicast MRB(s), if configured.

Upon initiating the resume procedure for SDT, AS security (both ciphering and integrity protection) is re-activated for SRB2 (if configured for SDT) and for SRB1. In addition, AS security is also re-activated (if security is configured) for all the DRBs configured for SDT. Further, the PDCP entities of SRB1 and PDCP entities of the radio bearers configured for SDT are re-established and resumed whilst the UE remains in RRC_INACTIVE state. Transmission and reception of data and/or signalling messages over radio bearers configured for SDT can happen whilst the UE is in RRC_INACTIVE state and SDT procedure is ongoing.

In response to a request to resume the RRC connection or in response to a resume procedure initiated for SDT, the network may resume the suspended RRC connection and send UE to RRC_CONNECTED, or reject the request to resume and send UE to RRC_INACTIVE (with a wait timer), or directly re-suspend the RRC connection and send UE to RRC_INACTIVE, or directly release the RRC connection and send UE to RRC_IDLE, or instruct the UE to initiate NAS level recovery (in this case the network sends an RRC setup message).

[TS 38.331, clause 5.3.8.3]

The UE shall:

1>	delay the following actions defined in this clause 60 ms from the moment the RRCRelease message was received or optionally when lower layers indicate that the receipt of the RRCRelease message has been successfully acknowledged, whichever is earlier;

1>	if the RRCRelease includes suspendConfig:

2>	reset MAC and release the default MAC Cell Group configuration, if any;

2>	apply the received suspendConfig except the received nextHopChainingCount;

2>	if the sdt-Config is configured:

3>	for each of the DRB in the sdt-DRB-List:

4>	consider the DRB to be configured for SDT;

3>	if sdt-SRB2-Indication is configured:

4>	consider the SRB2 to be configured for SDT;

3>	for each RLC bearer that is not suspended:

4>	re-establish the RLC entity as specified in TS 38.322 [4];

3>	for SRB2 (if it is resumed) and for SRB1:

4>	trigger the PDCP entity to perform SDU discard as specified in TS 38.323 [5];

3>	if sdt-MAC-PHY-CG-Config is configured:

4>	configure the PCell with the configured grant resources for SDT and instruct the MAC entity to start the cg-SDT-TimeAlignmentTimer;

…

2>	if the RRCRelease message with suspendConfig was received in response to an RRCResumeRequest or an RRCResumeRequest1:

3>	stop the timer T319 if running;

3>	in the stored UE Inactive AS context:

4>	replace the KgNB and KRRCint keys with the current KgNB and KRRCint keys;

4>	replace the nextHopChainingCount with the value of nextHopChainingCount received in the RRCRelease message;

4>	replace the cellIdentity with the cellIdentity of the cell the UE has received the RRCRelease message;

4>	if the suspendConfig contains the sl-UEIdentityRemote (i.e. the UE is a L2 U2N Remote UE):

…

4> else:

5>	replace the C-RNTI with the C-RNTI used in the cell (see TS 38.321 [3]) the UE has received the RRCRelease message;

5>	replace the physical cell identity with the physical cell identity of the cell the UE has received the RRCRelease message;

3>	replace the nextHopChainingCount with the value associated with the current KgNB;

3>	stop the timer T319a if running and consider SDT procedure is not ongoing;

2>	else:

3>	store in the UE Inactive AS Context the nextHopChainingCount received in the RRCRelease message, the current KgNB and KRRCint keys, the ROHC state, the EHC context(s), the UDC state, the stored QoS flow to DRB mapping rules, the application layer measurement configuration, the C-RNTI used in the source PCell, the cellIdentity and the physical cell identity of the source PCell, the spCellConfigCommon within ReconfigurationWithSync of the NR PSCell (if configured) and all other parameters configured except for:

-	parameters within ReconfigurationWithSync of the PCell;

-	parameters within ReconfigurationWithSync of the NR PSCell, if configured;

-	parameters within MobilityControlInfoSCG of the E-UTRA PSCell, if configured;

-	servingCellConfigCommonSIB;

-	…

NOTE 1c:	suspendConfig is not stored as part of UE Inactive AS Context, except for the fields explicitly specified.

3>	store any previously or subsequently received application layer measurement reports for which no segment, or full message, has been submitted to lower layers for transmission;

NOTE 2:	NR sidelink communication/discovery related configurations and logged measurement configuration are not stored as UE Inactive AS Context, when UE enters RRC_INACTIVE.

2>	suspend all SRB(s) and DRB(s) and multicast MRB(s), except SRB0 and broadcast MRBs;

2>	indicate PDCP suspend to lower layers of all DRBs and multicast MRBs;

2>	release the SRAP entity, if configured;

2>	if the t380 is included:

3>	start timer T380, with the timer value set to t380;

2>	if the RRCRelease message is including the waitTime:

3>	start timer T302 with the value set to the waitTime;

3>	inform upper layers that access barring is applicable for all access categories except categories '0' and '2';

2>	if T390 is running:

3>	stop timer T390 for all access categories;

3>	perform the actions as specified in 5.3.14.4;

2>	indicate the suspension of the RRC connection to upper layers;

2>	if the UE is capable of L2 U2N Remote UE:

3>	enter RRC_INACTIVE, and perform either cell selection as specified in TS 38.304 [20], or relay selection as specified in clause 5.8.15.3, or both;

2>	else:

3>	enter RRC_INACTIVE and perform cell selection as specified in TS 38.304 [20];

1>	else:

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with the release cause 'other'.

NOTE 3:	Whether to release the PC5 unicast link is left to L2 U2N Remote UE's implementation.

NOTE 4:	It is left to UE implementation whether to stop T430, if running, when going to RRC_INACTIVE.

[TS 38.331, clause 5.3.13.1]

The purpose of this procedure is to resume a suspended RRC connection, including resuming SRB(s), DRB(s) and multicast MRB(s) or perform an RNA update. This procedure is also used to initiate SDT in RRC_INACTIVE.

[TS 38.331, clause 5.3.13.1b]

A UE in RRC_INACTIVE initiates the resume procedure for SDT when all of the following conditions are fulfilled:

1>	the upper layers request resumption of RRC connection; and

1>	SIB1 includes sdt-ConfigCommon; and

1>	sdt-Config is configured; and

1>	all the pending data in UL is mapped to the radio bearers configured for SDT; and

1>	lower layers indicate that conditions for initiating SDT as specified in TS 38.321 [3] are fulfilled.

NOTE:	How the UE determines that all pending data in UL is mapped to radio bearers configured for SDT is left to UE implementation.

[TS 38.331, clause 5.3.13.2]

The UE initiates the procedure when upper layers or AS (when responding to RAN paging, upon triggering RNA updates while the UE is in RRC_INACTIVE, for NR sidelink communication/discovery/V2X sidelink communication as specified in clause 5.3.13.1a) requests the resume of a suspended RRC connection or requests the resume for initiating SDT as specified in clause 5.3.13.1b.

The UE shall ensure having valid and up to date essential system information as specified in clause 5.2.2.2 before initiating this procedure.

Upon initiation of the procedure, the UE shall:

…

1>	else:

2>	apply the default L1 parameter values as specified in corresponding physical layer specifications, except for the parameters for which values are provided in SIB1;

2>	apply the default SRB1 configuration as specified in 9.2.1;

2>	apply the default MAC Cell Group configuration as specified in 9.2.2;

1>	else:

2>	apply the CCCH configuration as specified in 9.1.1.2;

2>	apply the timeAlignmentTimerCommon included in SIB1;

1>	if sdt-MAC-PHY-CG-Config is configured:

2>	if the resume procedure is initiated in a cell that is different to the PCell in which the UE received the stored sdt-MAC-PHY-CG-Config:

3>	release the stored sdt-MAC-PHY-CG-Config;

3>	instruct the MAC entity to stop the cg-SDT-TimeAlignmentTimer, if it is running;

1>	if conditions for initiating SDT in accordance with 5.3.13.1b are fulfilled:

2>	consider the resume procedure is initiated for SDT;

2>	start timer T319a when the lower layers first transmit the CCCH message;

2>	consider SDT procedure is ongoing;

1> else:

2>	start timer T319;

2>	instruct the MAC entity to stop the cg-SDT-TimeAlignmentTimer, if it is running;

…

1>	initiate transmission of the RRCResumeRequest message or RRCResumeRequest1 in accordance with 5.3.13.3.

[TS 38.331, clause 5.3.13.3]

The UE shall set the contents of RRCResumeRequest or RRCResumeRequest1 message as follows:

…

1>	if the resume procedure is initiated for SDT:

2>	for each radio bearer that is configured for SDT and for SRB1:

3>	restore the RLC-BearerConfig associated with the RLC bearers of masterCellGroup and pdcp-Config from the UE Inactive AS context;

…

3>	re-establish PDCP entity for the radio bearer that is configured for SDT without triggering PDCP status report;

2>	resume all the radio bearers that are configured for SDT;

1>	submit the selected message RRCResumeRequest or RRCResumeRequest1 for transmission to lower layers.

NOTE 2:	Only DRBs with previously configured UP ciphering shall resume ciphering.

If lower layers indicate an integrity check failure while T319 is running or SDT procedure is ongoing, perform actions specified in 5.3.13.5.

If the UE is a RedCap UE and the RedCap-specific initial downlink BWP is not associated with CD-SSB, the UE may continue cell re-selection related measurements as well as cell re-selection evaluation, otherwise the UE shall continue cell re-selection related measurements as well as cell re-selection evaluation. If the conditions for cell re-selection are fulfilled, the UE shall perform cell re-selection as specified in 5.3.13.6.

[TS 38.331, clause 5.7.4.2]

A UE capable of SDT initiates this procedure when data and/or signalling mapped to radio bearers that are not configured for SDT becomes available during SDT (i.e. while SDT procedure is ongoing).

Upon initiating the procedure, the UE shall:

1>	if data and/or signalling mapped to radio bearers not configured for SDT becomes available during SDT (i.e., while SDT procedure is ongoing):

2>	if the UE did not transmit a UEAssistanceInformation message with nonSDT-DataIndication since the initiation of the current resume procedure for SDT:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide nonSDT-DataIndication.

[TS 38.331, clause 5.7.4.3]

The UE shall set the contents of the UEAssistanceInformation message as follows:

1>	if transmission of the UEAssistanceInformation message is initiated to indicate availability of data mapped to radio bearers not configured for SDT according to 5.7.4.2:

2>	include the nonSDT-DataIndication in the UEAssistanceInformation message;

2>	include and set the resumeCause according to the information received from the upper layers, if provided.

8.1.5.13.2.3	Test Description

8.1.5.13.2.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used in NR cell with modifications to SIB1 given in Table 8.1.5.13.2.3.3-1.

UE:

-	The UE does not have any stored SMS message.

Preamble:

-	The UE is in 5GS state 3N-A according to clause 4.4A.2 Table 4.4A.2-3 of TS 38.508-1 [4], registered with "SMS over NAS supported" as per test specific messages in table 8.1.5.13.2.3.3-2 and table 8.1.5.13.2.3.3-3 and test state parameter UE test loop mode B prepared according to clause 4.4A.3 Table 4.4A.3-1 of TS 38.508-1 [4].

8.1.5.13.2.3.2	Test procedure sequence

Table 8.1.5.13.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a CLOSE UE TEST LOOP message with UE test loop mode B selected and IP PDU delay in T_delay_modeB timer configured as defined in Table 8.1.5.13.2.3.3-4. | <-- | NR RRC: DLInformationTransfer TC: CLOSE UE TEST LOOP | - | - |
| 2 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message. | --> | NR RRC: ULInformationTransfer TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| 3 | The SS transmits one IP Packet to the UE on the DRB associated with the first PDU session on NR Cell 1. The IP Packet is less than sdt-DataVolumeThreshold. (Note 1) | - | - | - | - |
| 4 | The UE buffers the received IP Packet and starts T_delay_modeB timer. | - | - | - | - |
| 5 | The SS transmits an RRCRelease message with suspendConfig which includes SDT-CG-Config-r17 and sdt-DRB-List-r17 IEs. The default DRB in sdt-DRB-List-r17 is enabled for SDT. | <-- | NR RRC: RRCRelease | - | - |
| 6 | After some time when T_delay_modeB timer expires, buffered IP Packets are submitted in the same order as received (first-in-first-out). | - | - | - | - |
| 7 | UE transmits an RRCResumeRequest and the looped back IP Packet received in step 3 in a CG PUSCH occasion. | --> | NR RRC: RRCResumeRequest | - | - |
| 8 | Sending of a 160-character MO SMS is initiated at the UE via MMI or AT command. (non-SDT signalling arrives) | - | - | - | - |
| 9 | Check: Does the UE transmit an UEAssistanceInformation message containing nonSDT-DataIndication-r17 IE including resumeCause-r17? | --> | NR RRC: UEAssistanceInformation | 1 | P |
| 10 | The SS transmits RRCResume message as defined in Table 4.6.1-17 in TS 38.508-1 [4] to get UE to RRC_CONNECTED mode. | <-- | NR RRC: RRCResume | - | - |
| 11 | The UE transmits RRCResumeComplete message to SS and buffered MO SMS message | --> | NR RRC: RRCResumeComplete 5GMM: UL NAS TRANSPORT | - | - |
| 12 | The SS transmits a CP-ACK encapsulated in a Downlink NAS Transport message. | <-- | 5GMM: DL NAS TRANSPORT | - | - |
| Note 1: RLC PDU for IP packet is 97 bytes (RLC SDU is 94 bytes for 3 bytes RLC header and 95 bytes for 2 bytes RLC header) and sdt-DataVolumeThreshold is 100 bytes. Therefore, the size of RLC SDU is less than the sdt-DataVolumeThreshold. |  |  |  |  |  |

8.1.5.13.2.3.3	Specific message contents

Table 8.1.5.13.2.3.3-1: SIB1 (Preamble and all steps, Table 8.1.5.13.2.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-28 with condition SDT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sdt-ConfigCommon-r17 SEQUENCE { |  |  |  |
| sdt-RSRP-Threshold-r17 | 60 | (IE value – 156) dBm = - 96 dBm |  |
| sdt-LogicalChannelSR-DelayTimer-r17 | Not present |  |  |
| sdt-DataVolumeThreshold-r17 | byte100 |  |  |
| t319a-r17 | ms4000 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.13.2.3.3-2: REGISTRATION REQUEST (Preamble)

| Derivation Path: 38.508-1 [4] Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS update type |  |  |  |  |
| SMS requested |  | ‘1’B | SMS over NAS supported |  |

Table 8.1.5.13.2.3.3-3: REGISTRATION ACCEPT (Preamble)

| Derivation Path: 38.508-1 [4] Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration result |  |  |  |  |
| SMS allowed |  | ‘1’B | SMS over NAS allowed |  |

Table 8.1.5.13.2.3.3-4: CLOSE UE TEST LOOP (Step 1, Table 8.1.5.13.2.3.2-1)

| Derivation path: 38.508-1 [4] Table 4.7A-3 condition UE test loop mode B |  |  |
| --- | --- | --- |
| Information Element | Value/Remark | Comment |
| UE test loop mode B LB setup |  |  |
| IP PDU delay | 0 0 0 0 0 0 1 0 | 2 seconds |

Table 8.1.5.13.2.3.3-5: RRCRelease (Step 5, Table 8.1.5.13.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-16 with condition NR_RRC_INACTIVE and SDT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |
| suspendConfig SEQUENCE { |  |  |  |
| sdt-Config-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sdt-MAC-PHY-CG-Config-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cg-SDT-ConfigInitialBWP-NUL-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| pusch-Config-r17 CHOICE { |  |  |  |
| setup | PUSCH-Config | Derivation Path: TS 38.508-1 [4], Table 4.6.3-118 |  |
| } |  |  |  |
| configuredGrantConfigToAddModList-r17 SEQUENCE (SIZE (1..maxNrofConfiguredGrantConfig-r16)) OF ConfiguredGrantConfig { |  |  |  |
| ConfiguredGrantConfig | ConfiguredGrantConfig | Table 8.1.5.13.2.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cg-SDT-ConfigInitialBWP-DL-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| pdcch-Config-r17 | Not present |  |  |
| pdsch-Config-r17 CHOICE { |  |  |  |
| setup | PDSCH-Config | Derivation Path: TS 38.508-1 [4], Table 4.6.3-100 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cg-SDT-TimeAlignmentTimer-r17 | infinity | To ensure that the timer does not expire before the AT command is finished. |  |
| cg-SDT-RSRP-ThresholdSSB-r17 | 60 | (IE value – 156) dBm = - 96 dBm |  |
| cg-SDT-TA-ValiditationConfig-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cg-SDT-RSRP-ChangeThreshold-r17 | dB10 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.13.2.3.3-6: ConfiguredGrantConfig

Derivation Path: TS 38.508-1 [4], Table 4.6.3-26 with condition CG_Config_Type1

Table 8.1.5.13.2.3.3-7: RRCResumeRequest (Step 7, Table 8.1.5.13.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCResumeRequest ::= SEQUENCE { |  |  |  |
| rrcResumeRequest SEQUENCE { |  |  |  |
| resumeCause | mo-data |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.13.2.3.3-8: UEAssistanceInformation (Step 9, Table 8.1.5.13.2.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-30 with condition SDT |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEAssistanceInformation ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| ueAssistanceInformation SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonSDT-DataIndication-r17 SEQUENCE { |  |  |  |  |
| resumeCause-r17 |  | mo-SMS |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.5.13.2.3.3-9: DL NAS TRANSPORT (step 10, Table 8.1.5.13.2.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Payload container type |  | ‘0010’B | SMS |  |
| Payload container |  | CP-ACK |  |  |

##### 8.1.5.13.3	RRC SDT / CG based SDT / SDT-SRB2-Indication

8.1.5.13.3.1	Test Purpose (TP)

(1)

with { UE in NR RRC_INACTIVE state, CG based SDT is configured, SRB2 is configured for SDT and indicated to UE through sdt-SRB2-Indication }

ensure that {

when { UE has any NAS message to transmit in the UL }

then { UE shall transmit the UL NAS message through SRB2 }

}

8.1.5.13.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 38.331, clause 5.3.1.1, 5.3.8.3, 5.3.13.1, 5.3.13.1b, 5.3.13.2, 5.3.13.3, and TS 24.501 clause 5.3.1.4. Unless otherwise stated, these are Rel-17 requirements.

[TS 38.331, clause 5.3.1.1]

…

In response to a request to resume the RRC connection or in response to a resume procedure initiated for SDT, the network may resume the suspended RRC connection and send UE to RRC_CONNECTED, or reject the request to resume and send UE to RRC_INACTIVE (with a wait timer), or directly re-suspend the RRC connection and send UE to RRC_INACTIVE, or directly release the RRC connection and send UE to RRC_IDLE, or instruct the UE to initiate NAS level recovery (in this case the network sends an RRC setup message).

[TS 38.331, clause 5.3.8.3]

The UE shall:

…

1>	if the RRCRelease includes suspendConfig:

…

2>	if the sdt-Config is configured:

3>	for each of the DRB in the sdt-DRB-List:

4>	consider the DRB to be configured for SDT;

3>	if sdt-SRB2-Indication is configured:

4>	consider the SRB2 to be configured for SDT;

…

3>	if sdt-MAC-PHY-CG-Config is configured:

4>	configure the PCell with the configured grant resources for SDT and instruct the MAC entity to start the cg-SDT-TimeAlignmentTimer;

…

[TS 38.331, clause 5.3.13.1]

The purpose of this procedure is to resume a suspended RRC connection, including resuming SRB(s), DRB(s) and multicast MRB(s) or perform an RNA update. This procedure is also used to initiate SDT in RRC_INACTIVE.

[TS 38.331, clause 5.3.13.1b]

A UE in RRC_INACTIVE initiates the resume procedure for SDT when all of the following conditions are fulfilled:

1>	the upper layers request resumption of RRC connection; and

1>	SIB1 includes sdt-ConfigCommon; and

1>	sdt-Config is configured; and

1>	all the pending data in UL is mapped to the radio bearers configured for SDT; and

1>	lower layers indicate that conditions for initiating SDT as specified in TS 38.321 [3] are fulfilled.

NOTE:	How the UE determines that all pending data in UL is mapped to radio bearers configured for SDT is left to UE implementation.

[TS 38.331, clause 5.3.13.2]

The UE initiates the procedure when upper layers or AS (when responding to RAN paging, upon triggering RNA updates while the UE is in RRC_INACTIVE, for NR sidelink communication/discovery/V2X sidelink communication as specified in clause 5.3.13.1a) requests the resume of a suspended RRC connection or requests the resume for initiating SDT as specified in clause 5.3.13.1b.

The UE shall ensure having valid and up to date essential system information as specified in clause 5.2.2.2 before initiating this procedure.

Upon initiation of the procedure, the UE shall:

…

1>	else if the resumption of the RRC connection is triggered by upper layers:

…

2>	if the resumption occurs after release with redirect with mpsPriorityIndication:

3>	set the resumeCause to mps-PriorityAccess;

2>	else:

3>	set the resumeCause in accordance with the information received from upper layers;

…

1>	if sdt-MAC-PHY-CG-Config is configured:

2>	if the resume procedure is initiated in a cell that is different to the PCell in which the UE received the stored sdt-MAC-PHY-CG-Config:

3>	release the stored sdt-MAC-PHY-CG-Config;

3>	instruct the MAC entity to stop the cg-SDT-TimeAlignmentTimer, if it is running;

1>	if conditions for initiating SDT in accordance with 5.3.13.1b are fulfilled:

2>	consider the resume procedure is initiated for SDT;

2>	start timer T319a when the lower layers first transmit the CCCH message;

2>	consider SDT procedure is ongoing;

…

1>	initiate transmission of the RRCResumeRequest message or RRCResumeRequest1 in accordance with 5.3.13.3.

[TS 38.331, clause 5.3.13.3]

The UE shall set the contents of RRCResumeRequest or RRCResumeRequest1 message as follows:

…

1>	if the resume procedure is initiated for SDT:

2>	for each radio bearer that is configured for SDT and for SRB1:

3>	restore the RLC-BearerConfig associated with the RLC bearers of masterCellGroup and pdcp-Config from the UE Inactive AS context;

…

3>	re-establish PDCP entity for the radio bearer that is configured for SDT without triggering PDCP status report;

2>	resume all the radio bearers that are configured for SDT;

1>	submit the selected message RRCResumeRequest or RRCResumeRequest1 for transmission to lower layers.

NOTE 2:	Only DRBs with previously configured UP ciphering shall resume ciphering.

If lower layers indicate an integrity check failure while T319 is running or SDT procedure is ongoing, perform actions specified in 5.3.13.5.

…

[TS 24.501, clause 5.3.1.4]

…

The UE is in 5GMM-CONNECTED mode with RRC inactive indication when the UE is in:

a)	5GMM-CONNECTED mode over 3GPP access at the NAS layer; and

b)	RRC_INACTIVE state at the AS layer (see 3GPP TS 38.300 [27]).

Unless stated otherwise, the UE behaviour in 5GMM-CONNECTED mode with RRC inactive indication follows the UE behaviour in 5GMM-CONNECTED over 3GPP access, except that:

a)	the UE shall apply the mobility restrictions; and

b)	the UE shall perform the PLMN selection procedures

as in 5GMM-IDLE mode over 3GPP access.

The UE shall transition from 5GMM-CONNECTED mode over 3GPP access to 5GMM-CONNECTED mode with RRC inactive indication upon receiving an indication from the lower layers that the RRC connection has been suspended.

NOTE 1:	Any pending procedure or uplink data packet when receiving an indication from the lower layers that the RRC connection has been suspended, triggers a request to the lower layers to transition to RRC_CONNECTED state. This is also the case when the pending procedure or uplink data packet triggered a previous request to the lower layers to transition to RRC_CONNECTED state.

If the UE in 3GPP access is configured for eCall only mode as specified in 3GPP TS 31.102 [22] then:

-	if the UE with the N1 NAS signalling connection established for eCall over IMS moved to 5GMM-CONNECTED mode with RRC inactive indication, the UE shall start timer T3444; and

-	if the UE with the N1 NAS signalling connection established for a call to an HPLMN designated non-emergency MSISDN or URI for test or terminal reconfiguration service moved to 5GMM-CONNECTED mode with RRC inactive indication, the UE shall start timer T3445.

Upon:

a)	a trigger of a procedure which requires sending of a NAS message different from a REGISTRATION REQUEST message with the NG-RAN-RCU bit of the 5GS update type IE set to "UE radio capability update needed";

b)	an uplink user data packet to be sent for a PDU session with suspended user-plane resources;

c)	a trigger to request resources for 5G ProSe direct discovery over PC5 or 5G ProSe direct communication over PC5; or

d)	a trigger to request resources for V2X communication over PC5 (see 3GPP TS 23.287 [6C]);

the UE in 5GMM-CONNECTED mode with RRC inactive indication over 3GPP access shall request the lower layers to transition to RRC_CONNECTED state (see 3GPP TS 38.300 [27]).

NOTE 2:	If the UE supports Small Data Transmission (SDT) (see 3GPP TS 38.300 [27]), the following applies:

a)	if the UE due to pending uplink NAS messages or user data packets is requesting the lower layers to transition to RRC_CONNECTED state, but has not received a response from the lower layers, the UE can send the pending NAS messages or user data packets to the lower layers, and can receive multiple downlink NAS messages or multiple downlink user data packets from the lower layers while the UE remains in 5GMM-CONNECTED mode with RRC inactive indication over 3GPP access (i.e., without transitioning to 5GMM-CONNECTED mode). When the NAS layer triggers the transmission of pending uplink NAS messages or user data packets, and if the SDT is ongoing, the NAS layer will receive the response from the lower layers only after the SDT session has completed or failed;

b)	the NAS layer is not aware of the classification of NAS messages or the user data packets as belonging to the SDT session at the lower layers; and

c)	the setting of access category and the RRC establishment cause indicated to the lower layers when sending the pending uplink user data packets while the UE remains in 5GMM-CONNECTED mode with RRC inactive indication, is left to implementation.

…

The UE shall transition from 5GMM-CONNECTED mode with RRC inactive indication to 5GMM-CONNECTED mode over 3GPP access upon receiving an indication from the lower layers that the UE has transitioned to RRC_CONNECTED state (see 3GPP TS 38.300 [27]).

NOTE 3:	The AMF can be aware of the transition between 5GMM-CONNECTED mode and 5GMM-CONNECTED mode with RRC inactive indication for a UE (see 3GPP TS 23.502 [9]).

…

If the UE in 5GMM-CONNECTED mode with RRC inactive indication receives an indication from the lower layers that the RRC connection has been suspended, the UE shall stay in 5GMM-CONNECTED mode with RRC inactive indication. The UE shall re-initiate any pending procedure that had triggered the request to the lower layers to transition to RRC_CONNECTED state, if still needed.

8.1.5.13.3.3	Test Description

8.1.5.13.3.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used in NR cell with modifications to SIB1 given in Table 8.1.5.13.3.3.3-1.

UE:

-	The UE does not have any stored SMS message.

Preamble:

-	The UE is in 5GS state 3N-A according to clause 4.4A.2 Table 4.4A.2-3 of TS 38.508-1 [4], registered with "SMS over NAS supported". as per test specific messages in table 8.1.5.13.3.3.3-7 and table 8.1.5.13.3.3.3-8

8.1.5.13.3.3.2	Test procedure sequence

Table 8.1.5.13.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCRelease message with suspendConfig which includes SDT-CG-Config-r17 and sdt-SRB2-Indication-r17 IEs. | <-- | NR RRC: RRCRelease | - | - |
| 2 | Sending of a 160 character MO SMS is initiated at the UE via MMI or AT command. | <-- | - | - | - |
| 3 | The UE transmits an RRCResumeRequest using SRB0 | --> | NR RRC: RRCResumeRequest | - | - |
| 4 | Check: Does the UE transmit a CP-DATA containing an RP-DATA RPDU (SMS SUBMIT TPDU) encapsulated in an Uplink NAS transport message using SRB2? | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT | 1 | P |
| 5 | The SS transmits a CP-ACK encapsulated in a Downlink NAS Transport message. | <-- | NR RRC: DLInformationTransfer 5GMM: DL NAS TRANSPORT |  |  |
| 6 | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| Note: RRCResumeRequest and ULInformationTransfer messages (in step 3 & 4) are sent at the same MAC PDU |  |  |  |  |  |

8.1.5.13.3.3.3	Specific message contents

Table 8.1.5.13.3.3.3-1: SIB1 (Preamble and all steps, Table 8.1.5.13.3.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-28 with condition SDT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| sdt-ConfigCommon-r17 SEQUENCE { |  |  |  |
| sdt-RSRP-Threshold-r17 | 60 | (IE value – 156) dBm = - 96 dBm |  |
| sdt-LogicalChannelSR-DelayTimer-r17 | Not present |  |  |
| sdt-DataVolumeThreshold-r17 | byte200 |  |  |
| t319a-r17 | ms4000 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.13.3.3.3-2: RRCRelease (Step 1, Table 8.1.5.13.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-16 with condition NR_RRC_INACTIVE and SDT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |
| suspendConfig SEQUENCE { |  |  |  |
| sdt-Config-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sdt-SRB2-Indication-r17 | allowed |  |  |
| sdt-MAC-PHY-CG-Config-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cg-SDT-ConfigInitialBWP-NUL-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| pusch-Config-r17 CHOICE { |  |  |  |
| setup | PUSCH-Config | Derivation Path: TS 38.508-1 [4], Table 4.6.3-118 |  |
| } |  |  |  |
| configuredGrantConfigToAddModList-r17 SEQUENCE (SIZE (1..maxNrofConfiguredGrantConfig-r16)) OF ConfiguredGrantConfig { |  |  |  |
| ConfiguredGrantConfig | ConfiguredGrantConfig | Table 8.1.5.13.3.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cg-SDT-ConfigInitialBWP-DL-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| pdcch-Config-r17 | Not present |  |  |
| pdsch-Config-r17 CHOICE { |  |  |  |
| setup | PDSCH-Config | Derivation Path: TS 38.508-1 [4], Table 4.6.3-100 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cg-SDT-TimeAlignmentTimer-r17 | infinity | To ensure that the timer does not expire before the AT command is finished |  |
| cg-SDT-RSRP-ThresholdSSB-r17 | 60 | (IE value – 156) dBm = - 96 dBm |  |
| cg-SDT-TA-ValiditationConfig-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cg-SDT-RSRP-ChangeThreshold-r17 | dB10 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.13.3.3.3-3: ConfiguredGrantConfig

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-19 with condition CG_Config_Type1 and the following exceptions: |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConfiguredGrantConfig ::= SEQUENCE { |  |  |  |
| rrc-ConfiguredUplinkGrant SEQUENCE { |  |  |  |
| mcsAndTBS | 4 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.13.3.3.3-4: RRCResumeRequest (Step 3, Table 8.1.5.13.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCResumeRequest ::= SEQUENCE { |  |  |  |
| rrcResumeRequest SEQUENCE { |  |  |  |
| resumeCause | mo-data |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.13.3.3.3-5: UL NAS TRANSPORT (Step 4, Table 8.1.5.13.3.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Payload container type | ‘0010’B | SMS |  |
| Payload container | CP-DATA | RP-DATA RPDU |  |

Table 8.1.5.13.3.3.3-6: DL NAS TRANSPORT (Step 5, Table 8.1.5.13.3.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Payload container type | ‘0010’B | SMS |  |
| Payload container | CP-ACK |  |  |

Table 8.1.5.13.3.3.3-7: REGISTRATION REQUEST (Preamble)

| Derivation Path: 38.508-1 [4] Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS update type |  |  |  |  |
| SMS requested |  | ‘1’B | SMS over NAS supported |  |

Table 8.1.5.13.3.3.3-8: REGISTRATION ACCEPT (Preamble)

| Derivation Path: 38.508-1 [4] Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration result |  |  |  |  |
| SMS allowed |  | ‘1’B | SMS over NAS allowed |  |

##### 8.1.5.13.4	RRC MT SDT / CG based SDT / Success

8.1.5.13.4.1	Test Purpose (TP)

(1)

with { UE in NR RRC_INACTIVE state and CG-SDT resource is configured }

ensure that {

when {  UE receives a Paging message with MT-SDT Paging and DL RSRP is above mt-sdt-RSRP-Threshold configured and cg-SDT-TimeAlignmentTimer is running }

then { UE Initiates RRC Resume on CG resource and successfully completes MT SDT Procedure }

}

8.1.5.13.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331, clauses 5.3.2.3, 5.3.13.1b, and TS 38.321 clause 5.27.1. Unless otherwise stated these are Rel-18 requirements.

[TS 38.331 clause 5.3.2.3]

Upon receiving the Paging message by the UE or receiving PagingRecord from its connected L2 U2N Relay UE by a L2 U2N Remote UE, the UE shall:

1>	if in RRC_IDLE, for each of the PagingRecord, if any, included in the Paging message, or

1>	if in RRC_IDLE, for the PagingRecord, if any, included in the UuMessageTransferSidelink message received from the connected L2 U2N Relay UE:

2>	if the ue-Identity included in the PagingRecord matches the UE identity allocated by upper layers:

3>	if upper layers indicate the support of paging cause:

4>	forward the ue-Identity, accessType (if present) and paging cause (if determined) to the upper layers;

3>	else:

4>	forward the ue-Identity and accessType (if present) to the upper layers;

NOTE 1:	If the L2 U2N Relay UE supports the MUSIM feature, it can forward the paging cause to the connected L2 U2N Remote UE.

1>	if in RRC_INACTIVE, for each of the PagingRecord, if any, included in the Paging message, or

1>	if in RRC_INACTIVE, for the PagingRecord, if any, included in the UuMessageTransferSidelink message received from the connected L2 U2N Relay UE:

2>	if the ue-Identity included in the PagingRecord matches the UE's stored fullI-RNTI:

3>	if the UE is configured by upper layers with Access Identity 1:

4>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mps-PriorityAccess;

3>	else if the UE is configured by upper layers with Access Identity 2:

4>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mcs-PriorityAccess;

3>	else if the UE is configured by upper layers with one or more Access Identities equal to 11-15:

4>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to highPriorityAccess;

3>	else if mt-SDT indication was included in the Paging message and if the conditions for initiating SDT for a resume procedure initiated in response to RAN paging according to 5.3.13.1b are fulfilled:

4>	if pagingGroupList was not included in the Paging message; or:

4>	if pagingGroupList was included in the Paging message but the UE has not joined any MBS session(s) indicated by the TMGI(s) included in the pagingGroupList or:

4>	if pagingGroupList was included in the Paging message, the UE is configured to receive MBS multicast in RRC_INACTIVE, and inactiveReceptionAllowed was included for all the MBS session(s) indicated by the TMGI(s) that the UE has joined:

5>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mt-SDT:

4>	else:

5>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mt-Access;

3>	else:

4>	initiate the RRC connection resumption procedure according to 5.3.13 with resumeCause set to mt-Access;

NOTE 2:	If both conditions for initiating MT-SDT and MO-SDT according to 5.3.13.1b are fulfilled, UE may initiate RRC connection resumption procedure for MT-SDT or MO-SDT based on implementation.

NOTE 3:	A MUSIM UE may not initiate the RRC connection resumption procedure, e.g. when it decides not to respond to the Paging message due to UE implementation constraints as specified in TS 24.501 [23].

2>	else if the ue-Identity included in the PagingRecord matches the UE identity allocated by upper layers:

3>	if upper layers indicate the support of paging cause:

4>	forward the ue-Identity, accessType (if present) and paging cause (if determined) to the upper layers;

3>	else:

4>	forward the ue-Identity and accessType (if present) to the upper layers;

3>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause 'other';

1>	if in RRC_IDLE, for each TMGI included in pagingGroupList, if any, included in the Paging message:

2>	if the UE has joined an MBS session indicated by the TMGI included in the pagingGroupList:

3>	forward the TMGI to the upper layers;

[TS 38.331 clause 5.3.13.1b]

When requesting lower layers to check the conditions for initiating SDT, RRC indicates to lower layers whether the resume procedure is initiated for mobile originated or mobile terminated case.

A UE in RRC_INACTIVE initiates the resume procedure for SDT when all of the following conditions are fulfilled:

1>	for the resume procedure initiated by the upper layers (i.e. mobile originated case):

2>	SIB1 includes sdt-ConfigCommon; and

2>	sdt-Config is configured; and

2>	all the pending data in UL is mapped to the radio bearers configured for SDT; and

2>	for an (e)RedCap UE when RedCap-specific initial downlink BWP includes no CD-SSB, ncd-SSB-RedCapInitialBWP-SDT is configured; and

2>	lower layers indicate that conditions for initiating MO-SDT as specified in TS 38.321 [3] are fulfilled.

1>	for the resume procedure initiated in response to RAN paging (i.e. mobile terminated case):

2>	lower layers indicate that conditions for initiating MT-SDT as specified in TS 38.321 [3] are fulfilled.

NOTE:	How the UE determines that all pending data in UL is mapped to radio bearers configured for SDT is left to UE implementation.

[TS 38.321 clause 5.27.1]

The MAC entity may be configured by RRC with SDT and the SDT procedure may be initiated by RRC layer for MO-SDT or MT-SDT. The SDT procedure initiated for MO-SDT can be performed either by Random Access procedure with 2-step RA type or 4-step RA type (i.e., RA-SDT) or by configured grant Type 1 (i.e., CG-SDT). The SDT procedure initiated for MT-SDT cannot be performed by RA-SDT (i.e., RA-SDT is not applicable as specified in clause 5.1.1b), but can be performed either by Random Access procedure (i.e., with 2-step RA type or 4-step RA type) or by configured grant Type 1 (i.e., CG-SDT).

RRC configures the following parameters for SDT procedure:

-	sdt-DataVolumeThreshold: data volume threshold for the UE to determine whether to perform SDT procedure initiated for MO-SDT;

-	sdt-RSRP-Threshold: RSRP threshold for UE to determine whether to perform SDT procedure initiated for MO-SDT;

-	mt-SDT-RSRP-Threshold: RSRP threshold for UE to determine whether to perform SDT procedure initiated for MT-SDT;

-	cg-SDT-RSRP-ThresholdSSB: an RSRP threshold configured for SSB selection for CG-SDT;

-	cg-MT-SDT-MaxDurationToNextCG-Occasion: time threshold which is used by the UE to determine whether to perform CG-SDT for MT-SDT;

-	cg-SDT-MaxDurationToNextCG-Occasion: time threshold configured per logical channel which is used by the UE to determine whether to perform CG-SDT for MO-SDT;

-	sdt-BeamFailureRecoveryProhibitTimer: prohibit timer to avoid frequent triggering of Random Access procedure due to beam failure recovery during RA-SDT procedure or during MT-SDT procedure initiated by Random Access procedure.

The following UE variable is used for the SDT procedure:

-	MAX_DURATION_TO_NEXT_CG_OCCASION;

-	RSRP_THRESHOLD.

The MAC entity shall, if initiated by the upper layers for SDT procedure:

1>	if SDT procedure is initiated for MO-SDT as specified in TS 38.331 [5]:

2>	set the MAX_DURATION_TO_NEXT_CG_OCCASION to the shortest value of cg-SDT-MaxDurationToNextCG-Occasion, if configured, among all the logical channels configured with this parameter by upper layer and having data for transmission;

2>	set the RSRP_THRESHOLD to the value of sdt-RSRP-Threshold, if configured.

1>	else if SDT procedure is initiated for MT-SDT as specified in TS 38.331 [5]:

2>	set the MAX_DURATION_TO_NEXT_CG_OCCASION to the value of cg-MT-SDT-MaxDurationToNextCG-Occasion, if configured;

2>	if mt-SDT-RSRP-Threshold is configured:

3>	set the RSRP_THRESHOLD to the value of mt-SDT-RSRP-Threshold.

2>	else if sdt-RSRP-Threshold is configured:

3>	set the RSRP_THRESHOLD to the value of sdt-RSRP-Threshold.

1>	if the SDT procedure is initiated for MO-SDT as specified in TS 38.331 [5], and the data volume of the pending UL data across all RBs configured for SDT is less than or equal to sdt-DataVolumeThreshold, or if the SDT procedure is initiated for MT-SDT as specified in TS 38.331 [5]; and

NOTE 1:	For SDT procedure, the MAC entity also considers the suspended RBs configured with SDT for data volume calculation. It is up to the UE's implementation how the UE calculates the data volume for the suspended RBs. Size of the CCCH message is not considered for data volume calculation

1>	if the RSRP of the downlink pathloss reference is higher than RSRP_THRESHOLD or if RSRP_THRESHOLD is not set:

2>	if the Serving Cell is configured with supplementary uplink as specified in TS 38.331 [5]; and

2>	if the RSRP of the downlink pathloss reference is less than rsrp-ThresholdSSB-SUL:

3>	select the SUL carrier.

2>	else:

3>	select the NUL carrier.

2>	if CG-SDT is configured on the selected UL carrier, and TA for CG-SDT is valid according to clause 5.27.2 in the first available CG occasion for initial CG-SDT transmission with CCCH message according to clause 5.8.2; and

2>	if the SDT procedure is initiated for MO-SDT as specified in TS 38.331 [5], and, for each RB having data available for transmission, configuredGrantType1Allowed, if configured for CG-SDT, is configured with value true for the corresponding logical channel, or if the SDT procedure is initiated for MT-SDT as specified in TS 38.331 [5]; and

2>	if at least one SSB configured for CG-SDT with SS-RSRP above cg-SDT-RSRP-ThresholdSSB is available, and if either the time gap between the initiation of the SDT procedure and first available CG occasion for initial CG-SDT transmission with CCCH message according to clause 5.8.2 is less than MAX_DURATION_TO_NEXT_CG_OCCASION, or if the MAX_DURATION_TO_NEXT_CG_OCCASION is not set:

3>	indicate to the upper layers that the conditions for initiating SDT procedure are fulfilled;

3>	perform CG-SDT procedure on the selected UL carrier according to clause 5.8.2.

2>	else if a set of Random Access resources for RA-SDT is configured and can be selected according to clause 5.1.1b on the selected UL carrier on the BWP configured by initialUplinkBWP-RedCap, if configured for an (e)RedCap UE; otherwise, on the BWP configured by initialUplinkBWP; or

2>	if the SDT procedure is initiated for MT-SDT as specified in TS 38.331 [5]:

3>	if cg-SDT-TimeAlignmentTimer is running, consider cg-SDT-TimeAlignmentTimer as expired and perform the corresponding actions in clause 5.2;

3>	indicate to the upper layers that the conditions for initiating SDT procedure are fulfilled.

2>	else:

3>	indicate to the upper layers that the conditions for initiating SDT procedure are not fulfilled.

1>	else:

2>	indicate to the upper layers that the conditions for initiating SDT procedure are not fulfilled.

If Random Access procedure is selected above for SDT procedure initiated for MO-SDT or MT-SDT and after the Random Access procedure is successfully completed (see clause 5.1.6), the UE monitors PDCCH addressed to C-RNTI received in random access response until the SDT procedure is terminated. If CG-SDT is selected above and after the initial transmission for CG-SDT is performed, the UE monitors PDCCH addressed to C-RNTI as stored in UE Inactive AS context as specified in TS 38.331 [5] and CS-RNTI until the SDT procedure is terminated.

The MAC entity shall:

1>	if sdt-BeamFailureRecoveryProhibitTimer is configured and not running; and

1>	if RA-SDT procedure for MO-SDT or MT-SDT procedure initiated by Random Access procedure is ongoing:

2>	if SS-RSRP of the SSB selected in the last successfully completed Random Access procedure during ongoing RA-SDT procedure or ongoing MT-SDT procedure initiated by Random Access procedure is less than rsrp-ThresholdSSB:

3>	start the sdt-BeamFailureRecoveryProhibitTimer;

3>	initiate a Random Access procedure (see clause 5.1).

NOTE 1a:	It is up to UE implementation when to measure SSBs during ongoing RA-SDT procedure or ongoing MT-SDT procedure initiated by Random Access procedure. Based on implementation the UE need not initiate the Random Access procedure if there is no SSB with SS-RSRP above rsrp-ThresholdSSB. The UE uses rsrp-ThresholdSSB in Random Access configuration selected by the UE when RA-SDT or MT-SDT procedure was initiated.

NOTE 2:	When the UE determines if there is an SSB with SS-RSRP above cg-SDT-RSRP-ThresholdSSB or less than rsrp-ThresholdSSB, the UE uses the latest unfiltered L1-RSRP measurement.

8.1.5.13.4.3	Test description

8.1.5.13.4.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used in NR cell.

UE:

-	None.

Preamble:

-	The UE is in state 2N-A as defined in TS 38.508-1 [4], Table 4.4A.2-2 on NR Cell 1.

8.1.5.13.4.3.2	Test procedure sequence

Table 8.1.5.13.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a Paging message with MT-SDT indication. | <-- | NR RRC: Paging | - | - |
| 2 | Check: Does the UE transmit an RRCResumeRequest message on the CG resource with resume cause set to MT-SDT? | --> | NR RRC: RRCResumeRequest | 1 | P |
| 3 | The SS transmits a DL assignment using UE’s C-RNTI. | <-- | (DL Grant) | - | - |
| 4 | The SS transmits a DL MAC PDU containing a RLC PDU (Note1). | <-- | MAC PDU | - | - |
| 5 | Check: Does the UE transmit a HARQ ACK? | --> | HARQ ACK | 1 | P |
| 6 | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| Note 1: RLC PDU is 97 bytes (RLC SDU is 94 bytes for 3 bytes RLC header and 95 bytes for 2 bytes RLC header) and sdt-DataVolumeThreshold is 100 bytes. Therefore, the size of RLC SDU is less than the sdt-DataVolumeThreshold. |  |  |  |  |  |

8.1.5.13.4.3.3	Specific message contents

Table 8.1.5.13.4.3.3-1: RRCRelease (Preamble, Table 8.1.5.13.4.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-16 with condition NR_RRC_INACTIVE and SDT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCRelease ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |
| suspendConfig SEQUENCE { |  |  |  |
| sdt-Config-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sdt-DRB-List-r17 SEQUENCE (SIZE (0..maxDRB)) OF DRB-Identity { | 1 entry |  |  |
| DRB-Identity[1] | DRB-Identity using condition DRBj | Entry 1 j is the ID of the DRB established during the preamble which is allocated according to internal TTCN mapping |  |
| } |  |  |  |
| sdt-MAC-PHY-CG-Config-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cg-SDT-ConfigInitialBWP-NUL-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| pusch-Config-r17 CHOICE { |  |  |  |
| setup | PUSCH-Config | TS 38.508-1[4] Table 4.6.3-118: PUSCH-Config |  |
| } |  |  |  |
| configuredGrantConfigToAddModList-r17 SEQUENCE (SIZE (1..maxNrofConfiguredGrantConfig-r16)) OF ConfiguredGrantConfig { |  |  |  |
| ConfiguredGrantConfig[1] | ConfiguredGrantConfig | Table 8.1.5.13.4.3.3-2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cg-SDT-ConfigInitialBWP-SUL-r17 | Not present |  |  |
| cg-SDT-ConfigInitialBWP-DL-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| pdcch-Config-r17 | Not present |  |  |
| pdsch-Config-r17 CHOICE { |  |  |  |
| setup | PDSCH-Config | TS 38.508-1[4] Table 4.6.3-100: PDSCH-Config |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| cg-SDT-TimeAlignmentTimer-r17 | ms5120 | TimeAlignmentTimer(5.12s) is larger than IP PDU delay timer(2s) for Loop mode B. |  |
| cg-SDT-RSRP-ThresholdSSB-r17 | 60 | (IE value – 156) dBm = - 96 dBm | Preamble |
| cg-SDT-TA-ValiditationConfig-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cg-SDT-RSRP-ChangeThreshold-r17 | dB10 |  |  |
| } |  |  |  |
| } |  |  |  |
| cg-SDT-CS-RNTI-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.13.4.3.3-2: ConfiguredGrantConfig (Table 8.1.5.13.4.3.3-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-26, condition CG_Config_Type1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ConfiguredGrantConfig ::= SEQUENCE { |  |  |  |
| rrc-ConfiguredUplinkGrant SEQUENCE{ |  |  |  |
| frequencyDomainAllocation | BIT STRING (SIZE(18) | Equals to NBWPsize * (NBWPsize – LRB + 1) + (NBWPsize -1 - RBstart), where LRB = 24 PRB, RBstart = 0, NBWPsize is the size [PRBs] of the carrier bandwidth and contained in TS.38.508-1 [4] clause 6.2.3.1 | SCS15 AND BW5, SCS30 AND BW10 |
|  | BIT STRING (SIZE(18) | Equal to NBWPsize * (LRB-1) + RBstart), where LRB = 15 PRB, RBstart = 0, NBWPsize is the size [PRBs] of the carrier bandwidth and contained in TS.38.508-1 [4] clause 6.2.3.1 | BW3 |
|  | BIT STRING (SIZE(18) | Equal to NBWPsize * (LRB-1) + RBstart), where LRB = 24 PRB, RBstart = 0, NBWPsize is the size [PRBs] of the carrier bandwidth and contained in TS.38.508-1 [4] clause 6.2.3.1 |  |
| dmrs-SeqInitialization | 0 |  |  |
| mcsAndTBS | 0 |  |  |
|  | 2 |  | BW3 |
| } |  |  |  |
| } |  |  |  |
| Note: Configured UL grant for CG-SDT is 888 bits (LRBs & IMCS as per 38.523-3[3] annex B) is chosen to enable UE to transmit RRCResumeRequest and Loop backed RLC PDU in a MAC PDU ( 97 bytes for Loop backed RLC PDU + 2 bytes for MAC Sub Header + 6 bytes for RRCResumeRequest message + 2 bytes for MAC Sub Header + 4 bytes for BSR and Padding ) |  |  |  |

Table 8.1.5.13.4.3.3-3: RRC Resume Request (step 2, Table 8.1.5.13.4.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCResumeRequest ::= SEQUENCE { |  |  |  |
| rrcResumeRequest SEQUENCE { |  |  |  |
| resumeCause | mt-SDT-v1810 |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.1.5.14	Non-Terrestrial Network

##### 8.1.5.14.1	NR NTN / SIB19 / T430 with the timer value set to ntn-UlSyncValidityDuration / T430 Expiry

8.1.5.14.1.1	Test Purpose (TP)

(1)

with { UE has acquired SIB19 and in RRC_CONNECTED state and starts timer T430 with ntn-UlSyncValidityDuration }

ensure that {

when { T430 expires in RRC_CONNECTED state }

then { UE informs lower layers that the UL synchronization is lost and initiates the UL synchronization restoration procedure }

}

8.1.5.14.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS38.331 clauses 5.2.2.1, 5.2.2.4.21 , 5.2.2.6, 5.2.2.3.2

Unless otherwise stated these are Rel-17 requirements.

[38.331, clause 5.2.2.1]

Figure 5.2.2.1-1: System information acquisition

The UE applies the SI acquisition procedure to acquire the AS, NAS- and positioning assistance data information. The procedure applies to UEs in RRC_IDLE, in RRC_INACTIVE and in RRC_CONNECTED.

The UE in RRC_IDLE and RRC_INACTIVE shall ensure having a valid version of (at least) the MIB, SIB1 through SIB4, SIB5 (if the UE supports E-UTRA), SIB11 (if the UE is configured for idle/inactive measurements), SIB12 (if UE is capable of NR sidelink communication/discovery and is configured by upper layers to receive or transmit NR sidelink communication/discovery), and SIB13, SIB14 (if UE is capable of V2X sidelink communication and is configured by upper layers to receive or transmit V2X sidelink communication), SIB15 (if UE is configured by upper layers to report disaster roaming related information), SIB16 (if the UE is capable of slice-based cell reselection and the UE receives NSAG information for cell reselection from upper layer), SIB17 (if the UE is using TRS resources for power saving in RRC_IDLE and RRC_INACTIVE) and SIB19 (if UE is accessing NR via NTN access).

[38.331, clause 5.2.2.4.21]

Upon receiving SIB19, the UE in RRC_CONNECTED shall:

1>	start or restart T430 for serving cell with the timer value set to ntn-UlSyncValidityDuration for the serving cell from the subframe indicated by epochTime for the serving cell;

NOTE:	UE should attempt to re-acquire SIB19 before the end of the duration indicated by ntn-UlSyncValidityDuration and epochTime by UE implementation.

[38.331, clause 5.2.2.6]

The UE shall:

1>	if T430 for serving cell expires and if in RRC_CONNECTED:

2>	inform lower layers that UL synchronisation is lost;

2>	acquire SIB19 as defined in clause 5.2.2.3.2;

2>	upon successful acquisition of SIB19:

3>	inform lower layers when UL synchronisation is obtained;

NOTE:	The exact time when UL synchronisation is obtained (after SIB19 is acquired) is left to UE implementation, which can be from the subframe indicated by epochTime and optionally before the subframe indicated by epochTime.

[38.331, clause 5.2.2.3.2]

For SI message acquisition PDCCH monitoring occasion(s) are determined according to searchSpaceOtherSystemInformation. If searchSpaceOtherSystemInformation is set to zero, PDCCH monitoring occasions for SI message reception in SI-window are same as PDCCH monitoring occasions for SIB1 where the mapping between PDCCH monitoring occasions and SSBs is specified in TS 38.213[13]. If searchSpaceOtherSystemInformation is not set to zero, PDCCH monitoring occasions for SI message are determined based on search space indicated by searchSpaceOtherSystemInformation. PDCCH monitoring occasions for SI message which are not overlapping with UL symbols (determined according to tdd-UL-DL-ConfigurationCommon) are sequentially numbered from one in the SI window. The [x×N+K]th PDCCH monitoring occasion (s) for SI message in SI-window corresponds to the Kth transmitted SSB, where x = 0, 1, ...X-1, K = 1, 2, …N, N is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 and X is equal to CEIL(number of PDCCH monitoring occasions in SI-window/N). The actual transmitted SSBs are sequentially numbered from one in ascending order of their SSB indexes. The UE assumes that, in the SI window, PDCCH for an SI message is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB and thus the selection of SSB for the reception SI messages is up to UE implementation.

When acquiring an SI message, the UE shall:

1>	determine the start of the SI-window for the concerned SI message as follows:

2>	if the concerned SI message is configured in the schedulingInfoList:

3>	for the concerned SI message, determine the number n which corresponds to the order of entry in the list of SI messages configured by schedulingInfoList in si-SchedulingInfo in SIB1;

3>	determine the integer value x = (n – 1) × w, where w is the si-WindowLength;

3>	the SI-window starts at the slot #a, where a = x mod N, in the radio frame for which SFN mod T = FLOOR(x/N), where T is the si-Periodicity of the concerned SI message and N is the number of slots in a radio frame as specified in TS 38.211 [16];

2>	else if the concerned SI message is configured in the schedulingInfoList2;

3>	determine the integer value x = (si-WindowPosition -1) × w, where w is the si-WindowLength;

3>	the SI-window starts at the slot #a, where a = x mod N, in the radio frame for which SFN mod T = FLOOR(x/N), where T is the si-Periodicity of the concerned SI message and N is the number of slots in a radio frame as specified in TS 38.211 [16];

2>	else if the concerned SI message is configured in the posSchedulingInfoList and offsetToSI-Used is not configured:

3>	create a concatenated list of SI messages by appending the posSchedulingInfoList in posSI-SchedulingInfo in SIB1 to schedulingInfoList in si-SchedulingInfo in SIB1;

3>	for the concerned SI message, determine the number n which corresponds to the order of entry in the concatenated list;

3>	determine the integer value x = (n – 1) × w, where w is the si-WindowLength;

3>	the SI-window starts at the slot #a, where a = x mod N, in the radio frame for which SFN mod T = FLOOR(x/N), where T is the posSI-Periodicity of the concerned SI message and N is the number of slots in a radio frame as specified in TS 38.211 [16];

2>	else if the concerned SI message is configured by the posSchedulingInfoList and offsetToSI-Used is configured:

3>	determine the number m which corresponds to the number of SI messages with an associated si-Periodicity of 8 radio frames (80 ms), configured by schedulingInfoList in SIB1;

3>	for the concerned SI message, determine the number n which corresponds to the order of entry in the list of SI messages configured by posSchedulingInfoList in SIB1;

3>	determine the integer value x = m × w + (n – 1) × w, where w is the si-WindowLength;

3>	the SI-window starts at the slot #a, where a = x mod N, in the radio frame for which SFN mod T = FLOOR(x/N) +8, where T is the posSI-Periodicity of the concerned SI message and N is the number of slots in a radio frame as specified in TS 38.211 [16];

1>	receive the PDCCH containing the scheduling RNTI, i.e. SI-RNTI in the PDCCH monitoring occasion(s) for SI message acquisition, from the start of the SI-window and continue until the end of the SI-window whose absolute length in time is given by si-WindowLength, or until the SI message was received;

1>	if the SI message was not received by the end of the SI-window, repeat reception at the next SI-window occasion for the concerned SI message in the current modification period;

1>	if all the SIB(s) and/or posSIB(s) requested in DedicatedSIBRequest message have been acquired:

2>	stop timer T350, if running;

NOTE 1:	The UE is only required to acquire broadcasted SI message if the UE can acquire it without disrupting unicast or MBS multicast data reception, i.e. the broadcast and unicast/MBS multicast beams are quasi co-located.

NOTE 2:	The UE is not required to monitor PDCCH monitoring occasion(s) corresponding to each transmitted SSB in SI-window.

NOTE 3:	If the concerned SI message was not received in the current modification period, handling of SI message acquisition is left to UE implementation.

NOTE 4:	A UE in RRC_CONNECTED may stop the PDCCH monitoring during the SI window for the concerned SI message when the requested SIB(s) are acquired.

NOTE 5:	A UE capable of NR sidelink communication/discovery and configured by upper layers to perform NR sidelink communication/discovery on a frequency, may acquire SIB12 or SystemInformationBlockType28 from a cell other than current serving cell (for RRC_INACTIVE or RRC_IDLE) or current PCell (for RRC_CONNECTED), if SIB12 of current serving cell (for RRC_INACTIVE or RRC_IDLE) or current PCell (for RRC_CONNECTED) does not provide configuration for NR sidelink communication/discovery for the frequency, and if the other cell providing configuration for NR sidelink communication/discovery for the frequency meets the S-criteria as defined in TS 38.304 [20] or TS 36.304 [27].

1>	perform the actions for the acquired SI message as specified in clause 5.2.2.4.

8.1.5.14.1.3	Test description

8.1.5.14.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 as specified in TS 38.508-1 [4] Table 4.4.2.1.

-	System information combination NR-29 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used.

UE:

-	The pre-configured UE location is defined in TS 38.508-1 [4] Clause 4.5C.

Preamble

-	The UE is in state Switched OFF (state 0N-B) as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1.

8.1.5.14.1.3.2	Test procedure sequence

Table 8.1.5.14.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Power on UE. | - | - | - | - |
| 2-19 | Steps 2 to 19 of the registration procedure described in TS 38.508-1 [4] table 4.5.2.2-2 | - | - | - | - |
| 20 | IF connected without release is On AND Test Loop Function=On THEN the SS transmits a CLOSE UE TEST LOOP message to enter the UE test loop mode. The CLOSE UE TEST LOOP is using the associated condition for the test loop. | <-- | NR RRC: DLInformationTransfer TC: CLOSE UE TEST LOOP | - | - |
| 21 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message to confirm that loopback entities for the radio bearer(s) have been created and loop back is activated. | --> | NR RRC: ULInformationTransfer TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| 22 | SS stops broadcasting SIB19. Note : The sysinfo combination is changed to NR-2 | - | - | - | - |
| 23 | SS start a timer T1 with value = T430-(((Current SFN- epochsfn)*10)/1000) | - | - | - | - |
| 23A | SS waits for duration of T1-2s(Note 1) | - | - | - | - |
| 24 | SS stops UL grant allocation | - | - | - | - |
| 25 | The SS transmits one IP packet to the UE on the DRB associated with the first PDU session on NR Cell 1. | <-- | PDCP PDU DATA #0 | - | - |
| 26 | Wait until expiry of T1. | - | - | - | - |
| 26A | SS checks if UE sends Scheduling Request for 2 sec (UE should not send Scheduling Request when UL Sync is lost) | - | - | 1 | F |
| 27 | SS resumes broadcasting SIB19. Note : The sysinfo combination is changed to NR-29 | - | - | - | - |
| 28 | The SS starts the UL grant transmission allowing the UE to return the IP packet as received at step 25. | - | - | - | - |
| 29 | Check: Does the UE loop back the IP packet received at step 25 in NR Cell 1? | --> | PDCP PDU DATA #0 | 1 | P |
| Note 1: 2s is chosen so that the SS has enough time to stop UL grant at Step 24 and schedule transmission of DL Data at Step 25 before T1 expiry |  |  |  |  |  |

8.1.5.14.1.3.3	Specific message contents

Table 8.1.5.14.1.3.3-1: SIB19 for NR Cell1(preamble and all steps)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-18C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 { |  |  |  |
| sfn-r17 | 0 |  |  |
| subframeNR-r17 | 0 |  |  |
| } |  |  |  |
| ntn-UlSyncValidityDuration-r17 | s20 | Value of T430 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.14.1.3.3-2: SchedulingRequest-Config (Preamble)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-155 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequestConfig ::= SEQUENCE { |  |  |  |
| schedulingRequestToAddModList SEQUENCE (SIZE(1..maxNrofSR-ConfigPerCellGroup)) OF SchedulingRequestToAddMod { | 1 entry |  |  |
| SchedulingRequestToAddMod[1] SEQUENCE { |  | entry 1 |  |
| sr-ProhibitTimer | ms64 |  |  |
| sr-TransMax | n64 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.1.5.14.2	NR NTN Enhancements / SIB19 / Satellite Hard Switch with Resync

8.1.5.14.2.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state and t-Service and SatSwitchWithReSync are broadcasted in SIB19 }

ensure that {

when{ upon the time indicated by t-Service }

then{ UE acquire target satellite indicated by ntn-Config in SatSwitchWithReSync }

}

8.1.5.14.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331 clauses 5.7.19 and TS 38.321 clause 5.2a. Unless otherwise stated these are Rel-18 requirements.

[TS 38.331, clause 5.7.19]

A UE capable of hard satellite switch with resynchronization in RRC_CONNECTED initiates the procedure when SatSwitchWithReSync and t-Service are included in SIB19.

Upon initiating the procedure, the UE shall:

1>	if t-ServiceStart is included in SIB19 and the UE supports soft satellite switch with resynchronization:

2>	start acquiring DL synchronization with the SpCell served by the satellite indicated by ntn-Config in SatSwitchWithReSync between the time indicated by t-ServiceStart and the time indicated by t-Service for the serving cell;

1>	upon the time indicated by t-Service:

2>	stop timer T430 if running;

2>	inform lower layers that UL synchronisation is lost due to satellite switch with resynchronization;

2>	synchronise to the DL of the SpCell served by the satellite indicated by ntn-Config in SatSwitchWithReSync, if the UE has not previously synchronized to the DL of the SpCell;

2>	start timer T430 with the timer value set to ntn-UlSyncValidityDuration from the subframe indicated by epochTime in ntn-Config in SatSwitchWithReSync;

2>	inform lower layers when UL synchronisation is obtained.

NOTE:	UE should attempt to re-acquire SIB19 after satellite switch with resynchronization. The exact time is left to UE implementation.

[TS 38.321, clause 5.2a]

The MAC entity shall for each Serving Cell:

1>	if an indication of uplink synchronization has been received from upper layers (see clauses 5.2.2.6 and 5.7.19 of TS 38.331 [5]):

2>	if indication of uplink synchronization is received after indication of uplink synchronization loss due to satellite switch with re-synchronization (see clause 5.7.19 of TS 38.331 [5]):

3>	set NTA value (as defined in TS 38.211 [8]) to zero for PTAG;

3>	indicate to lower layers a Differential Koffset with value zero.

2>	allow uplink transmission on the Serving Cell.

1>	if an indication of uplink synchronization loss is received from upper layers (see clause 5.2.2.6 and 5.7.19 of TS 38.331 [5]):

2>	if uplink synchronization loss is due to satellite switch with re-synchronization (see clause 5.7.19 of TS 38.331 [5]):

3>	not perform any uplink transmission on the Serving Cell.

2>	else:

3>	flush all HARQ buffers;

3>	not perform any uplink transmission on the Serving Cell.

NOTE:	The MAC entity suspends all UL operations (e.g. stop RACH, SR, and UL HARQ operation) after receiving the indication of an uplink synchronization loss and resumes the operation when receiving an indication of uplink synchronization.

8.1.5.14.2.3	Test description

8.1.5.14.2.3.1	Pre-test conditions

System Simulator:

NR Cell 1 is serving cell connected to the source satellite and NR Cell 2 is an intra-freq cell with same PCI connected to the target satellite

System information combination NR-29 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

-	The pre-configured UE location is defined in TS 38.508-1 [4] Clause 4.5C.

Preamble:

-	The UE is in 3N-A on NR Cell 1 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3 and Test Mode Activated according to 38.508-1 [4] Table 4.4A.2-1 with UE test loop mode B is established IP PDU delay set to FFS seconds.

8.1.5.14.2.3.2	Test procedure sequence

Table 8.1.5.14.2.3.2-1 for FR1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions for preamble, while columns marked "T1" to be applied subsequently in the Main behaviour. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.5.14.2.3.2-1: Cell configuration changes over time for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remarks |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | "Off" | Power level “Off” is defined in TS 38.508-1 Table 6.2.2.1-3 |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -88 |  |

Table 8.1.5.14.2.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits a downlink assignment including the C-RNTI assigned to the UE | <-- | (PDCCH (C-RNTI)) |  |  |
| 2 | SS transmits in the indicated downlink assignment a RLC PDU in a MAC PDU. on NR Cell 1 | <-- | MAC PDU |  |  |
| 3 | The SS re-adjusts the power level to row "T1" in table 8.1.5.14.2.3.2-1 | - | - |  |  |
| 4 | Wait for FFS ms (t-Service+) to allow the UE to switch to Target Satellite | - | - | - | - |
| 5 | Check: Does the UE transmit a MAC PDU containing Loop backed PDU on expiry of IP PDU delay on NR Cell 2? | --> | MAC PDU (containing 1 MAC sub PDU containing RLC SDU) | 1 | P |
| 6 | The SS transmits an RRCRelease message | <-- | NR RRC: RRCRelease | - | - |

8.1.5.14.2.3.3	Specific message contents

Table 8.1.5.14.2.3.3-1: PhysCellId for NR Cell 2 (preamble and all steps, Table 8.1.5.14.2.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-105 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysCellId | Set according to table 4.4.2-2 for the NR Cell 1 | NR Cell1 and NR Cell2 to have same PCI |  |

Table 8.1.5.14.2.3.3-2: SIB1 for NR Cell 2 (preamble and all steps, Table 8.1.5.14.2.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| servingCellConfigCommon | ServingCellConfigCommonSIB | Table 8.1.5.14.2.3.3-3 |  |
| } |  |  |  |

Table 8.1.5.14.2.3.3-3: ServingCellConfigCommonSIB (Table 8.1.5.14.2.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| uplinkConfigCommon | UplinkConfigCommonSIB | Table 8.1.5.14.2.3.3-4 |  |
| } |  |  |  |

Table 8.1.5.14.2.3.3-4: UplinkConfigCommonSIB (Table 8.1.5.14.2.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-202 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkConfigCommonSIB SEQUENCE { |  |  |  |
| timeAlignmentTimerCommon | TimeAlignmentTimer |  |  |
| } |  |  |  |

Table 8.1.5.14.2.3.3-5: SIB19 for NR Cell 1 (Preamble and all steps, Table 8.1.5.14.2.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-18C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| distanceThresh-r17 | FFS |  |  |
| movingReferenceLocation-r18 | FFS |  |  |
| t-Service | FFS |  |  |
| satSwitchWithReSync-r18 SEQUENCE { |  |  |  |
| ntn-Config-r18 | NTN-Config | Table 8.1.5.14.2.3.3-8 for NR Cell 2 |  |
| t-ServiceStart-r18 | Not Present |  |  |
| ssb-TimeOffset-r18 | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.14.2.3.3-6: SIB19 for NR Cell 2 (Preamble and all steps, Table 8.1.5.14.2.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-18C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r18 | NTN-Config | Table 8.1.5.14.2.3.3-8 |  |
| distanceThresh-r17 | FFS |  |  |
| movingReferenceLocation-r18 | FFS |  |  |
| } |  |  |  |

Table 8.1.5.14.2.3.3-7: NTN-Config (Table 8.1.5.14.2.3.3-6,7)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-84C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NTN-Config-r17 | FFS |  |  |

Table 8.1.5.14.2.3.3-8: CLOSE UE TEST LOOP (Preamble)

| Derivation path: TS 36.508 [7] table 4.7A-3 condition UE test loop mode B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE test loop mode B LB setup |  |  |  |
| IP PDU delay | FFS |  |  |

##### 8.1.5.14.3	NR NTN Enhancements / SIB19 / Satellite Soft Switch with Resync

8.1.5.14.3.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state  and t-ServiceStart, t-Service, ssb-TimeOffset, SatSwitchWithReSync are broadcasted in SIB19 }

ensure that {

when{ t-ServiceStart starts }

then{ UE acquires target satellite indicated by ntn-Config in SatSwitchWithReSync before t-Service expiry}

}

8.1.5.14.3.2	Conformance requirements

Same as 8.1.5.14.2,2

8.1.5.14.3.3	Test description

8.1.5.14.3.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is serving cell connected to the source satellite and NR Cell 2 is an intra-freq cell with same PCI connected to the target satellite

-	System information combination NR-29 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

-	The pre-configured UE location is defined in TS 38.508-1 [4] Clause 4.5C.

Preamble:

-	The UE is in 3N-A on NR Cell 1 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3 and Test Mode Activated according to 38.508-1 [4] Table 4.4A.2-1 with UE test loop mode B is established IP PDU delay set to FFS seconds.

8.1.5.14.3.3.2	Test procedure sequence

Table 8.1.5.14.3.3.3-1 for FR1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions for preamble, while columns marked "T1" to be applied subsequently in the Main behaviour. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.5.14.3.3.2-1: Cell configuration changes over time for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remarks |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | "Off" | Power level “Off” is defined in TS 38.508-1 Table 6.2.2.1-3 |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -88 |  |

Table 8.1.5.14.3.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits a downlink assignment including the C-RNTI assigned to the UE | <-- | (PDCCH (C-RNTI)) |  |  |
| 2 | SS transmits in the indicated downlink assignment a RLC PDU in a MAC PDU. on NR Cell 1 | <-- | MAC PDU |  |  |
| 3 | The SS re-adjusts the power level to row "T1" in table 8.1.5.14.2.3.2-1 | - | - |  |  |
| 4 | Wait for FFS ms (After t-ServiceStart and before t-Service) to allow the UE to switch to Target Satellite | - | - | - | - |
| 5 | Check: Does the UE transmit a MAC PDU containing Loop backed PDU on expiry of IP PDU delay on NR Cell 2? | --> | MAC PDU (containing 1 MAC sub PDU containing RLC SDU) | 1 | P |
| 6 | The SS transmits an RRCRelease message | <-- | NR RRC: RRCRelease | - | - |

8.1.5.14.3.3.3	Specific message contents

Same as 8.1.5.14.2.3.3 except

Table 8.1.5.14.3.3.3-1: SIB19 for NR Cell 1 (Preamble and all steps, Table 8.1.5.14.3.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-18C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| t-Service | FFS |  |  |
| distanceThresh-r17 | FFS |  |  |
| movingReferenceLocation-r18 | FFS |  |  |
| satSwitchWithReSync-r18 SEQUENCE { |  |  |  |
| ntn-Config-r18 | NTN-Config | Table 8.1.5.14.3.3.3-2 for NR Cell 2 |  |
| t-ServiceStart-r18 | FFS |  |  |
| ssb-TimeOffset-r18 | 10 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.14.3.3.3-2: NTN-Config (Table 8.1.5.14.3.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-84C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NTN-Config-r17 | FFS |  |  |

Table 8.1.5.14.3.3.3-3: CLOSE UE TEST LOOP (Preamble)

| Derivation path: TS 36.508 [7] table 4.7A-3 condition UE test loop mode B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE test loop mode B LB setup |  |  |  |
| IP PDU delay | FFS |  |  |  |

### 8.1.6	SON and MDT support for NR

#### 8.1.6.1	Intra NR MDT

##### 8.1.6.1.1	Immediate MDT

###### 8.1.6.1.1.1	Immediate MDT / Measurement reporting / Location information

8.1.6.1.1.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and measurement with event A2 configured with includeLocationInfo included in the reportConfig }

ensure that {

when { Entry condition for event A2 is met and detailed location information that has not been reported is available }

then { UE sends MeasurementReport message with locationInfo included }

}

(2)

with { UE in RRC_CONNECTED state and measurement configured for event A2 }

ensure that {

when { Exit condition for event A2 is met }

then { UE stops sending MeasurementReport message }

}

8.1.6.1.1.1.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 38.331 clause 5.3.5.3, 5.5.4.1, 5.5.4.3, 5.3.5.9, 6.3.4 and 5.5.5.1.

[TS 38.331, clause 5.3.5.3]

The UE shall perform the following actions upon reception of the RRCReconfiguration, or upon execution of the conditional reconfiguration (CHO or CPC):

...

1>	if the RRCReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

...

[TS 38.331, clause 5.5.4.1]

If AS security has been activated successfully, the UE shall:

1>	for each measId included in the measIdList within VarMeasConfig:

2>	if the corresponding reportConfig includes a reportType set to eventTriggered or periodical:

3>	if the corresponding measObject concerns NR:

4>	if the eventA1 or eventA2 is configured in the corresponding reportConfig:

5>	consider only the serving cell to be applicable;

4>	if the eventA3 or eventA5 is configured in the corresponding reportConfig:

5>	if a serving cell is associated with a measObjectNR and neighbours are associated with another measObjectNR, consider any serving cell associated with the other measObjectNR to be a neighbouring cell as well;

4>	if corresponding reportConfig includes reportType set to periodical; or

4>	for measurement events other than eventA1 or eventA2:

5>	if useAllowedCellList is set to true:

6>	consider any neighbouring cell detected based on parameters in the associated measObjectNR to be applicable when the concerned cell is included in the allowedCellsToAddModList defined within the VarMeasConfig for this measId;

5>	else:

6>	consider any neighbouring cell detected based on parameters in the associated measObjectNR to be applicable when the concerned cell is not included in the excludedCellsToAddModList defined within the VarMeasConfig for this measId;

...

2>	if the reportType is set to eventTriggered and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig, while the VarMeasReportList does not include a measurement reporting entry for this measId (a first cell triggers the event):

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	if useT312 is set to true in reportConfig for this event:

4>	if T310 for the corresponding SpCell is running; and

4>	if T312 is not running for corresponding SpCell:

5>	start timer T312 for the corresponding SpCell with the value of T312 configured in the corresponding measObjectNR;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	else if the reportType is set to eventTriggered and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells not included in the cellsTriggeredList for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig (a subsequent cell triggers the event):

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	if useT312 is set to true in reportConfig for this event:

4>	if T310 for the corresponding SpCell is running; and

4>	if T312 is not running for corresponding SpCell:

5>	start timer T312 for the corresponding SpCell with the value of T312 configured in the corresponding measObjectNR;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	else if the reportType is set to eventTriggered and if the leaving condition applicable for this event is fulfilled for one or more of the cells included in the cellsTriggeredList defined within the VarMeasReportList for this measId for all measurements after layer 3 filtering taken during timeToTrigger defined within the VarMeasConfig for this event:

3>	remove the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	if reportOnLeave is set to true for the corresponding reporting configuration:

4>	initiate the measurement reporting procedure, as specified in 5.5.5;

3>	if the cellsTriggeredList defined within the VarMeasReportList for this measId is empty:

4>	remove the measurement reporting entry within the VarMeasReportList for this measId;

4>	stop the periodical reporting timer for this measId, if running;

2>	else if the reportType is set to eventTriggered and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable transmission resource pools for all measurements taken during timeToTrigger defined for this event within the VarMeasConfig, while the VarMeasReportList does not include an measurement reporting entry for this measId (a first transmission resource pool triggers the event):

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned transmission resource pool(s) in the poolsTriggeredList defined within the VarMeasReportList for this measId;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	else if the reportType is set to eventTriggered and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable transmission resource pools not included in the poolsTriggeredList for all measurements taken during timeToTrigger defined for this event within the VarMeasConfig (a subsequent transmission resource pool triggers the event):

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned transmission resource pool(s) in the poolsTriggeredList defined within the VarMeasReportList for this measId;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	else if the reportType is set to eventTriggered and if the leaving condition applicable for this event is fulfilled for one or more applicable transmission resource pools included in the poolsTriggeredList defined within the VarMeasReportList for this measId for all measurements taken during timeToTrigger defined within the VarMeasConfig for this event:

3>	remove the concerned transmission resource pool(s) in the poolsTriggeredList defined within the VarMeasReportList for this measId;

3>	if the poolsTriggeredList defined within the VarMeasReportList for this measId is empty:

4>	remove the measurement reporting entry within the VarMeasReportList for this measId;

4>	stop the periodical reporting timer for this measId, if running

...

[TS 38.331, clause 5.5.4.3]

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A2-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A2-2, as specified below, is fulfilled;

1>	for this measurement, consider the serving cell indicated by the measObjectNR associated to this event.

Inequality A2-1 (Entering condition)

Ms + Hys < Thresh

Inequality A2-2 (Leaving condition)

Ms – Hys > Thresh

The variables in the formula are defined as follows:

Ms is the measurement result of the serving cell, not taking into account any offsets.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigNR for this event).

Thresh is the threshold parameter for this event (i.e. a2-Threshold as defined within reportConfigNR for this event).

Ms is expressed in dBm in case of RSRP, or in dB in case of RSRQ and RS-SINR.

Hys is expressed in dB.

Thresh is expressed in the same unit as Ms.

[TS 38.331, clause 5.3.5.9]

The UE shall:

…

1>	if the received otherConfig includes the obtainCommonLocation:

2>	include available detailed location information for any subsequent measurement report or any subsequent RLF report and SCGFailureInformation;

[TS 38.331, clause 6.3.4]

obtainCommonLocation Requests the UE to attempt to have detailed location information available using GNSS. NR configures the field if includeCommonLocationInfo is configured for one or more measurements.

[TS 38.331, clause 5.5.5.1]

…

For the measId for which the measurement reporting procedure was triggered, the UE shall set the measResults within the MeasurementReport message as follows:

1>	set the measId to the measurement identity that triggered the measurement reporting;

…

1>	if there is at least one applicable neighbouring cell to report:

…

1>	if the includeCommonLocationInfo is configured in the corresponding reportConfig for this measId and detailed location information that has not been reported is available, set the content of commonLocationInfo of the locationInfo as follows:

2>	include the locationTimestamp;

2>	include the locationCoordinate, if available;

2>	include the velocityEstimate, if available;

2>	include the locationError, if available;

2>	include the locationSource, if available;

2>	if available, include the gnss-TOD-msec,

…

1>	increment the numberOfReportsSent as defined within the VarMeasReportList for this measId by 1;

1>	stop the periodical reporting timer, if running;

1>	if the numberOfReportsSent as defined within the VarMeasReportList for this measId is less than the reportAmount as defined within the corresponding reportConfig for this measId:

2>	start the periodical reporting timer with the value of reportInterval as defined within the corresponding reportConfig for this measId;

1>	else:

2>	if the reportType is set to periodical or cli-Periodical:

3>	remove the entry within the VarMeasReportList for this measId;

3>	remove this measId from the measIdList within VarMeasConfig;

8.1.6.1.1.1.3	Test description

8.1.6.1.1.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

Preamble:

-	The UE's positioning engine (e.g. standalone GNSS receiver) should be provided with any necessary stimulus to allow it to provide the position.  This shall be done by use of the test function Update UE Location Information defined in TS 38.509 [6], if supported by the UE according to pc_UpdateUE_LocationInformation. Otherwise, or in addition any other suitable method may also be used.

-	The UE is in state 3N-A as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1.

8.1.6.1.1.1.3.2	Test procedure sequence

Table 8.1.6.1.1.1.3.2-1 and Table 8.1.6.1.1.1.3.2-1a illustrate the downlink power levels to be applied for NR Cell 1 at various time instants of the test execution. Row marked "T0" denotes the initial conditions after the preamble, while rows marked "T1" and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.6.1.1.1.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -78 | Power level is such that Ms > Thresh + Hys |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -98 | Power level is such that entry condition for event A2 is satisfied Ms + Hys < Thresh |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -78 | Power level is such that exit condition for event A2 is satisfied Ms - Hys > Thresh |

Table 8.1.6.1.1.1.3.2-1a: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -82 | Power level is such that Ms > Thresh + Hys |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -100 | Power level is such that entry condition for event A2 is satisfied Ms + Hys < Thresh |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -82 | Power level is such that exit condition for event A2 is satisfied Ms - Hys > Thresh |

Table 8.1.6.1.1.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message including measConfig to setup intra NR measurement and reporting for event A2 with includeLocationInfo configured. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | SS re-adjusts the SS/PBCH EPRE level according to row "T1" in Table 8.1.6.1.1.1.3.2-1 or Table 8.1.6.1.1.1.3.2-1a. | - | - | - | - |
| 4 | Check: Does the UE transmit a MeasurementReport message to report event A2 with the UE commonLocationInfo included? | --> | NR RRC: MeasurementReport | 1 | P |
| 5 | SS re-adjusts the SS/PBCH EPRE level according to row "T2" in Table 8.1.6.1.1.1.3.2-1 or Table 8.1.6.1.1.1.3.2-1a. | - | - | - | - |
| 6 | Wait and ignore MeasurementReport messages for 5 s to allow change of power levels for NR Cell 1. | - | - | - | - |
| 7 | Check: Does the UE attempt to transmit MeasurementReport message within the next 10s?? | - |  | 2 | F |

8.1.6.1.1.1.3.3	Specific message contents

Table 8.1.6.1.1.1.3.3-1: RRCReconfiguration (step 1 Table 8.1.6.1.1.1.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition NR_MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| otherConfig-v1610 SEQUENCE { |  |  |  |
| obtainCommonLocation-r16 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.1.1.3.3-2: MeasConfig (Table 8.1.6.1.1.1.3.3-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1.. maxNrofObjectId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR(57) | Thres=57(-100dBm ≤ SS-RSRP<-99dBm) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig | ReportConfigNR-EventA2 |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1.. maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.1.1.3.3-3: ReportConfigNR-EventA2 (Table 8.1.6.1.1.1.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-142 with condition EVENT_A2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA2 SEQUENCE { |  |  | EVENT_A2 |
| a2-Threshold CHOICE { |  |  |  |
| rsrp | 69 | -88dBm ≤ SS-RSRP<-87dBm | FR1 |
|  | 65 + Delta(NRfs) | SS RSRP corresponding to -91dBm | FR2 |
| } |  |  |  |
| hysteresis | 0 | 0 dB | FR1/FR2 |
| } |  |  |  |
| } |  |  |  |
| reportAmount | r1 |  |  |
| includeCommonLocationInfo-r16 | True |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Delta(NRfs) is derived based on calibration procedure defined in the TS 38.508-1 [4], clause 6.1.3.3 |  |  |  |

Table 8.1.6.1.1.1.3.3-4: MeasurementReport (steps 4, Table 8.1.6.1.1.1.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-5A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| measurementReport SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  | Step 4 |
| measResultServingMOList SEQUENCE { | 1 entry |  |  |
| servCellId | ServCellIndex of NR Cell 1 |  |  |
| measResultServingCell SEQUENCE { |  |  |  |
| physCellId | Physical CellID of the NR Cell 1 |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not present |  |  |
|  | (0..127) |  | pc_ss_SINR_Meas |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| locationInfo-r16 SEQUENCE { |  |  |  |
| commonLocationInfo-r16 | Any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

###### 8.1.6.1.1.2	Immediate MDT / Measurement / Latency metrics for UL PDCP Packet Delay per DRB

8.1.6.1.1.2.1	Test Purpose (TP)

(1)

with {UE supporting UL PDCP Packet Delay per DRB measurement and in RRC_CONNECTED state}

ensure that {

when {UE receives RRCReconfiguration containing ReportConfigNR IE with ul-DelayValueConfig IE set to “setup”}

then {UE sends the RRCReconfigurationComplete message}

}

(2)

with {UE in RRC_CONNECTED and measurement configured for UL PDCP Packet Delay per DRB and having DRBs established}

ensure that {
  when {UE determines that UL PDCP SDUs per DRB}

then {UE sends the MeasurementReport message with ul-PDCP-DelayResultList for corresponding DRB}

}

(3)

with { UE in RRC_CONNECTED and measurement configured for UL PDCP Packet Delay per DRB}

ensure that {

when {UE receives RRCReconfiguration containing ReportConfigNR IE with ul-DelayConfig IE set to “release”)

then {UE stops monitoring the UL PDCP Packet Delay per DRB}

}

8.1.6.1.1.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331, clause 5.5.3.1, 5.5.4.1, 5.5.5.1 & TS 38.314, clause 4.3.1.1

[TS 38.331, clause 5.5.3.1]

…

The UE shall:

…

1>	for each measId included in the measIdList within VarMeasConfig:

…

2>	if the ul-DelayValueConfig is configured for the associated reportConfig:

3>	ignore the measObject;

3>	for each of the configured DRBs, configure the PDCP layer to perform corresponding average UL PDCP packet delay measurement per DRB;

…

[TS 38.331, clause 5.5.4.1]

If AS security has been activated successfully, the UE shall:

1>	for each measId included in the measIdList within VarMeasConfig:

…

2>	if the corresponding reportConfig includes the ul-DelayValueConfig:

3>	initiate the measurement reporting procedure, as specified in 5.5.5, immediately after a first measurement result is provided by all lower layers of the associated DRB identity;

…

[TS 38.331, clause 5.5.5.1]

…

For the measId for which the measurement reporting procedure was triggered, the UE shall set the measResults within the MeasurementReport message as follows:

…

1>	if average uplink PDCP delay values are available:

2>	set the ul-PDCP-DelayValueResultList to include the corresponding average uplink PDCP delay values;

…

[TS 38.314, clause 4.3.1.1]

The objective of this measurement performed by UE is to measure Packet Delay in Layer PDCP for QoS verification of MDT or for the QoS monitoring as defined in TS 23.501.

Protocol Layer: PDCP

Table 4.3.1.1-1: Definition for UL PDCP Packet Average Delay per DRB per UE

| Definition | PDCP Packet Delay in the UL per DRB. This measurement refers to PDCP queuing delay for DRBs in the UE, which captures the delay from packet arrival at PDCP upper SAP until the UL grant to transmit the packet is available, which has included the delay the UE gets resources granted (from sending SR/RACH to get the first grant). The measurement is done separately per DRB. Detailed Definition: MT,drbid=∀itDelivi, drbid-tArrivali, drbidIT, where explanations can be found in the table 4.3.1.1-2 below. |
| --- | --- |

NOTE:	UE measures UL PDCP queueing delay at DRB level. It is up to gNB to convert DRB level delay to QoS level delay with the assumption that all QoS flows mapped to the same DRB get the same QoS treatment, and it is up to gNB to calculate QoS level delay if multiple DRBs mapped with the same QoS.

Table 4.3.1.1-2: Parameter description for UL PDCP Packet Average Delay per DRB per UE

| M(T,drbid) | PDCP average delay in the UL per DRB, averaged during time period 𝑇. Unit: 0.1 ms. PDCP average delay in the UL per DRB is 1s if the actual value is larger than 1s. |
| --- | --- |
| tArrival(i) | The point in time when the UL PDCP SDU i arrivals at PDCP upper SAP. |
| tDeliv(i) | The point in time when the UL MAC PDU k including the first part of UL PDCP SDU i is scheduled for transmission. |
| i | A UL PDCP SDU that is received by the PDCP during time period 𝑇. |
| I(T) | Total number of UL PDCP SDUs received during time period 𝑇. |
| T | Time Period during which the measurement is performed |
| drbid | The identity of the measured DRB. |

8.1.6.1.1.2.3	Test description

8.1.6.1.1.2.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

UE:

-	None

Preamble:

-	The UE is in 5GS state 3N-A with one PDU session active according to TS 38.508-1 [4], clause 4.4A.3 Table 4.4A.3-1.and using the message condition UE TEST LOOP MODE B active. 3 DRBs are configured where DRB#j is defined as default DRB.

8.1.6.1.1.2.3.2	Test procedure sequence

Table 8.1.6.1.1.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message including MeasConfig to setup ul-DelayValueConfig | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message | --> | NR RRC: RRCReconfigurationComplete | 1 | P |
|  | EXCEPTION: In parallel with steps 3-9, parallel behaviour defined in table 8.1.6.1.1.2.3.2-3 is executed to receive periodic measurement report. |  |  |  |  |
| 3 | The SS transmits UL grant to the UE at every 5ms in PDCCH occasion |  |  |  |  |
| 4 | The SS transmits an IP Packet on the data radio bearer associated with the dedicated 5GC bearer context #2 |  |  |  |  |
| 5 | The UE loop backs the IP packet received in step 4 on DRB#j+1 |  |  |  |  |
| 5A | Check: Does the UE transmit a MeasurementReport message containing UL-PDCP-DelayValueResult with averageDelay set to non zero value for DRB#j+1 and zero for DRB# j+2? | --> | NR RRC: MeasurementReport | 2 | P |
| 6 | The SS transmits an IP Packet on the data radio bearer associated with the dedicated 5GC bearer context #3 |  |  |  |  |
| 7 | The UE loop backs the IP packet received in step 6 on DRB# j+2 |  |  |  |  |
| 8 | Check: Does the UE transmit a MeasurementReport message containing UL-PDCP-DelayValueResult for DRB# j+1 and with with averageDelay set to non zero value DRB# j+2 and zero for DRB#j+1? | --> | MeasurementReport | 2 | P |
| 9 | The SS transmits an RRCReconfiguration message including MeasConfig to release the ul-DelayValueConfig measurement | <-- | NR RRC: RRCReconfiguration | - | - |
| 10 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete |  |  |
| - | EXCEPTION: In parallel with steps 11-14, parallel behaviour defined in table 8.1.6.1.1.2.3.2-2 is executed. | - | - | - | - |
| 11 | The SS transmits an IP Packet on the data radio bearer associated with the dedicated 5GC bearer context #2 |  |  |  |  |
| 12 | The UE loop backs the IP packet received in step 11 on DRB# j+1 |  |  |  |  |
| 13 | The SS transmits an IP Packet on the data radio bearer associated with the dedicated 5GC bearer context #3 |  |  |  |  |
| 14 | The UE loop backs the IP packet received in step 13 on DRB#j+2 |  |  |  |  |
| 15 | Void | - | - | - | - |

Table 8.1.6.1.1.2.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE transmit a MeasurementReport message containing UL-PDCP-DelayValueResult? | --> | MeasurementReport | 3 | F |

Table 8.1.6.1.1.2.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmit a MeasurementReport message | --> | MeasurementReport | - | - |

8.1.6.1.1.2.3.3	Specific message contents

Table 8.1.6.1.1.2.3.3-1: RRCReconfiguration (step 1 and 9, Table 8.1.6.1.1.2.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition NR_MEAS

Table 8.1.6.1.1.2.3.3-2: MeasConfig (Table 8.1.6.1.1.2.3.3-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1.. maxNrofObjectId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR(57) | Thres=57(-100dBm ≤ SS-RSRP<-99dBm) |  |
|  | MeasObjectNR(56 + Delta(NRfs)) | SS RSRP corresponding to -100dBm |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig | ReportConfigNR- PERIODICAL |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1.. maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.1.2.3.3-3: ReportConfigNR- PERIODICAL (Table 8.1.6.1.1.2.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-142 with condition PERIODICAL, MDT_DELAY |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| ul-DelayValueConfig-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  | Step1 |
| delay-DRBlist SEQUENCE (SIZE(1..maxDRB)) OF DRB-Identity { | 2 entries |  |  |
| DRB-Identity[1] | j+1 | entry 1 |  |
| DRB-Identity[2] | j+2 | entry 2 |  |
| } |  |  |  |
| } |  |  |  |
| Release | NULL |  | Step9 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.1.2.3.3-4: MeasurementReport (steps 5A and 8, Table 8.1.6.1.1.2.3.2-1 and step 1, Table 8.1.6.1.1.2.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-5A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| measurementReport SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServingMOList SEQUENCE { | 1 entry |  |  |
| servCellId | ServCellIndex of NR Cell 1 |  |  |
| measResultServingCell SEQUENCE { |  |  |  |
| physCellId | Physical CellID of the NR Cell 1 |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ul-PDCP-DelayValueResultList-r16 SEQUENCE (SIZE (1..maxDRB)) OF UL-PDCP-DelayValueResult-r16{ | 2 entries | Step 5A, Table 8.1.6.1.1.2.3.2-1 |  |
| drb-Id-r16[1] | j+1 | entry 1 |  |
| averageDelay-r16[1] | Any value other than 0 |  |  |
| drb-Id-r16[2] | j+2 | entry 2 |  |
| averageDelay-r16[2] | 0 |  |  |
| } |  |  |  |
| ul-PDCP-DelayValueResultList-r16 SEQUENCE (SIZE (1..maxDRB)) OF UL-PDCP-DelayValueResult-r16{ | 2 entries | Step8, Table 8.1.6.1.1.2.3.2-1 |  |
| drb-Id-r16[1] | j+1 | entry 1 |  |
| averageDelay-r16[1] | 0 |  |  |
| drb-Id-r16[2] | j+2 | entry 2 |  |
| averageDelay-r16[2] | Any value other than 0 |  |  |
| } |  |  |  |
| ul-PDCP-DelayValueResultList-r16 | Present | Step 1, Table 8.1.6.1.1.2.3.2-2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.1.2.3.3-5: RRCReconfiguration (Preamble, Table 8.1.6.1.1.2.3.3-1)

| Derivation Path: TS 38.508-1 [4], table 4.6.1-13 with condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig-3DRBs |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig-3DRBs | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.1.2.3.3-6: RadioBearerConfig-3DRBs (Table 8.1.6.1.1.2.3.3-4)

| Derivation Path: TS 38.508-1 [4], table 4.6.3-132 and condition SRB2 and DRB1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 3 entries |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config::= SEQUENCE { |  |  |  |
| pdu-Session | PDU session ID | Same as the PDU session ID in PDU SESSION ESTABLISHMENT REQUEST |  |
| defaultDRB | true |  |  |
| mappedQoS-FlowsToAdd SEQUENCE { |  |  |  |
| QFI | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| drb-Identity | DRB-Identity with Condition DRBj | j is the ID of the DRB established during the preamble which is allocated according to internal TTCN mapping |  |
| } |  |  |  |
| } |  |  |  |
| DRB-ToAddMod[2] SEQUENCE |  | entry 2 |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config SEQUENCE { |  |  |  |
| pdu-Session | PDU session ID | Same as the PDU session ID in PDU SESSION ESTABLISHMENT REQUEST |  |
| defaultDRB | false |  |  |
| mappedQoS-FlowsToAdd SEQUENCE { |  |  |  |
| QFI | 5 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| drb-Identity | DRB-Identity with Condition DRBj+1 |  |  |
| } |  |  |  |
| } |  |  |  |
| DRB-ToAddMod[3] SEQUENCE |  | entry 3 |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config SEQUENCE { |  |  |  |
| pdu-Session | PDU session ID | Same as the PDU session ID in PDU SESSION ESTABLISHMENT REQUEST |  |
| defaultDRB | false |  |  |
| mappedQoS-FlowsToAdd SEQUENCE { |  |  |  |
| QFI | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| drb-Identity | DRB-Identity with Condition DRBj+2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.1.2.3.3-7: CellGroupConfig-3DRBs (Table 8.1.6.1.1.2.3.3-4)

| Derivation Path: TS 38.508-1 [4], table 4.6.3-19 and condition SRB2_DRB1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-BearerConfig { | 4 entries |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig with condition SRB2 | entry 1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions AM and DRBj | entry 2 |  |
| RLC-BearerConfig[3] | RLC-BearerConfig with conditions AM and DRBj+1 | entry 3 |  |
| RLC-BearerConfig[3] | RLC-BearerConfig with conditions AM and DRBj+2 | entry 4 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.1.2.3.3-8: PDU SESSION ESTABLISHMENT ACCEPT (Preamble) (Table 8.1.6.1.1.2.3.3-1)

| Derivation Path: TS 38.508-1, table 4.7.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDU session ID | The same as the PDU session ID in PDU SESSION ESTABLISHMENT REQUEST |  |  |
| Authorized QoS rules | 3 entries |  |  |
| QoS rule [1] | Reference QoS rule #1 as defined in Table 4.8.2.1-1. | QFI=1 |  |
| QoS rule [2] | Reference QoS rule #4 as defined in Table 4.8.2.1-4 except DQR bit set to ‘0’B. | QFI=2 |  |
| QoS rule [3] | Reference QoS rule #5 as defined in Table 4.8.2.1-5. | QFI=5 |  |
| Mapped EPS Bearer contexts | Not Present |  |  |
| Authorized QoS flow descriptions | 3 entries |  |  |
| QoS flow [1] | Reference QoS flow #1 as defined in Table8.1.5.4.1.3.3-7. | QFI=1 |  |
| QoS flow [2] | Reference QoS flow #2 as defined in Table8.1.5.4.1.3.3-8. | QFI=2 |  |
| QoS flow [3] | Reference QoS flow #3 as defined in Table 4.8.2.3-3. | QFI=5 |  |

##### 8.1.6.1.2	Logged MDT

###### 8.1.6.1.2.1	Logged MDT / RRC_IDLE / Logging and reporting / Intra-frequency measurement

8.1.6.1.2.1.1	Test Purpose (TP)

(1)

with { UE in NR RRC_IDLE state camping normally on an NR cell where logged measurement is configured without areaConfiguration and the UE is able to detect an NR intra-frequency cell }

ensure that {

when { T330 is running }

then { UE is logging serving cell idle mode measurements and Intra-frequency neighbouring cell measurements }

}

(2)

with { UE in NR RRC_IDLE state and UE has one or more logged Intra-frequency neighbouring cell measurement entries stored in VarLogMeasReport }

ensure that {

when { receiving RRCSetup message }

then { UE includes the logMeasAvailable IE in the RRCSetupComplete message }

}

(3)

with { UE in NR RRC_CONNECTED state and UE has one or more logged Intra-frequency neighbouring cell measurement entries stored in VarLogMeasReport }

ensure that {

when { receiving UEInformationRequest message }

then { UE transmits UEInformationResponse messages with absoluteTimeStamp set equal to the value configured when the logged measurement configuration was received and a relativeTimeStamp for each logged measurement indicating the elapsed time since the logged measurement configuration was received }

}

(4)

with { UE in NR RRC_CONNECTED state and UE has one or more logged Intra-frequency neighbouring cell measurement entries stored in VarLogMeasReport }

ensure that {

when { receiving UEInformationRequest message }

then { UE transmits UEInformationResponse messages with traceReference, traceRecordingSessionRef and tce-Id being set to same values as configured when the logged measurement configuration was received }

}

8.1.6.1.2.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clauses 5.3.3.4, 5 5.5a.1.3, 5.5a.3.2, 5.7.10.3 and 6.2.2. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.3.3.4]

The UE shall perform the following actions upon reception of the RRCSetup:

…

1>	set the content of RRCSetupComplete message as follows:

…

2>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable in the RRCSetupComplete message;

2>	if the UE has Bluetooth logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailableBT in the RRCSetupComplete message;

2>	if the UE has WLAN logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailableWLAN in the RRCSetupComplete message;

…

[TS 38.331, clause 5.5a.1.3]

Upon receiving the LoggedMeasurementConfiguration message the UE shall:

1>	discard the logged measurement configuration as well as the logged measurement information as specified in 5.5a.2;

1>	store the received loggingDuration, reportType and areaConfiguration, if included, in VarLogMeasConfig;

1>	if the LoggedMeasurementConfiguration message includes plmn-IdentityList:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN as well as the PLMNs included in plmn-IdentityList;

1>	else:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN;

1>	store the received absoluteTimeInfo, traceReference, traceRecordingSessionRef, and tce-Id in VarLogMeasReport;

1>	store the received bt-NameList, if included, in VarLogMeasConfig;

1>	store the received wlan-Namelist, if included, in VarLogMeasConfig;

1>	store the received sensor-Namelist, if included, in VarLogMeasConfig;

1>	start timer T330 with the timer value set to the loggingDuration;

[TS 38.331, clause5.5a.3.2]

While T330 is running, the UE shall:

1>	perform the logging in accordance with the following:

2>	if the reportType is set to periodical in the VarLogMeasConfig:

3>	if the UE is camping normally on an NR cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and, if the cell is part of the area indicated by areaConfiguration if configured in VarLogMeasConfig:

4>	perform the logging at regular time intervals, as defined by the loggingInterval in the VarLogMeasConfig;

2>	else if the reportType is set to eventTriggered, and eventType is set to outOfCoverage:

3>	perform the logging at regular time intervals as defined by the loggingInterval in VarLogMeasConfig only when the UE is in any cell selection state;

3>	perform the logging immediately upon transitioning from the any cell selection state to the camped normally state;

2>	else if the reportType is set to eventTriggered and eventType is set to eventL1:

3>	if the UE is camping normally on an NR cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and, if the cell is part of the area indicated by areaConfiguration if configured in VarLogMeasConfig;

4>	perform the logging at regular time intervals as defined by the loggingInterval in VarLogMeasConfig only when the conditions indicated by the eventL1 are met;

2>	when performing the logging:

3>	set the relativeTimeStamp to indicate the elapsed time since the moment at which the logged measurement configuration was received;

3>	if detailed location information became available during the last logging interval, set the content of the locationInfo as in 5.3.3.7:

3>	if the UE is in any cell selection state (as specified in TS 38.304 [20]):

4>	set anyCellSelectionDetected to indicate the detection of no suitable or no acceptable cell found;

4>	set the servCellIdentity to indicate global cell identity of the last logged cell that the UE was camping on;

4>	set the measResultServingCell to include the quantities of the last logged cell the UE was camping on;

3>	else:

4>	set the servCellIdentity to indicate global cell identity of the cell the UE is camping on;

4>	set the measResultServingCell to include the quantities of the cell the UE is camping on;

4>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements that became available during the last logging interval for at most the following number of neighbouring cells: 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies per RAT and according to the following:

5>	for each neighbour cell included, include the optional fields that are available;

NOTE:	The UE includes the latest results of the available measurements as used for cell reselection evaluation in RRC_IDLE or RRC_INACTIVE, which are performed in accordance with the performance requirements as specified in TS 38.133 [14].

2>	when the memory reserved for the logged measurement information becomes full, stop timer T330 and perform the same actions as performed upon expiry of T330, as specified in 5.5a.1.4.

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if the logMeasReportReq is present and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3>	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3>	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3>	include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3>	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3>	include the logMeasInfoList and set it to include one or more entries from VarLogMeasReport starting from the entries logged first;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

3>	if the VarLogMeasReport includes one or more additional logged Bluetooth measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailableBT;

3>	if the VarLogMeasReport includes one or more additional logged WLAN measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailableWLAN;

…

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1.

[TS 38.331, clause 6.2.2]

–	LoggedMeasurementConfiguration

The LoggedMeasurementConfiguration message is used to perform logging of measurement results while in RRC_IDLE or RRC_INACTIVE. It is used to transfer the logged measurement configuration for network performance optimisation.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

LoggedMeasurementConfiguration message

-- ASN1START

-- TAG-LOGGEDMEASUREMENTCONFIGURATION-START

LoggedMeasurementConfiguration-r16 ::=  SEQUENCE {

criticalExtensions                      CHOICE {

loggedMeasurementConfiguration-r16      LoggedMeasurementConfiguration-r16-IEs,

criticalExtensionsFuture                SEQUENCE {}

}

}

LoggedMeasurementConfiguration-r16-IEs ::=  SEQUENCE {

traceReference-r16                          TraceReference-r16,

traceRecordingSessionRef-r16                OCTET STRING (SIZE (2)),

tce-Id-r16                                  OCTET STRING (SIZE (1)),

absoluteTimeInfo-r16                        AbsoluteTimeInfo-r16,

areaConfiguration-r16                       AreaConfiguration-r16                    OPTIONAL,  --Need R

plmn-IdentityList-r16                       PLMN-IdentityList2-r16                   OPTIONAL,  --Need R

bt-NameList-r16                             SetupRelease {BT-NameList-r16}           OPTIONAL,  --Need M

wlan-NameList-r16                           SetupRelease {WLAN-NameList-r16}         OPTIONAL,  --Need M

sensor-NameList-r16                         SetupRelease {Sensor-NameList-r16}       OPTIONAL,  --Need M

loggingDuration-r16                         LoggingDuration-r16,

reportType                                  CHOICE {

periodical                                  LoggedPeriodicalReportConfig-r16,

eventTriggered                              LoggedEventTriggerConfig-r16,

...

},

lateNonCriticalExtension                    OCTET STRING                             OPTIONAL,

nonCriticalExtension                        SEQUENCE {}                              OPTIONAL

}

LoggedPeriodicalReportConfig-r16 ::=            SEQUENCE {

loggingInterval-r16                             LoggingInterval-r16,

...

}

LoggedEventTriggerConfig-r16 ::=                SEQUENCE {

eventType-r16                                   EventType-r16,

loggingInterval-r16                             LoggingInterval-r16,

...

}

EventType-r16 ::= CHOICE {

outOfCoverage     NULL,

eventL1           SEQUENCE {

l1-Threshold      MeasTriggerQuantity,

hysteresis        Hysteresis,

timeToTrigger     TimeToTrigger

},

...

}

-- TAG-LOGGEDMEASUREMENTCONFIGURATION-STOP

-- ASN1STOP

LoggedMeasurementConfiguration field descriptions

absoluteTimeInfo Indicates the absolute time in the current cell.

areaConfiguration Used to restrict the area in which the UE performs measurement logging to cells broadcasting either one of the included cell identities or one of the included tracking area codes/ frequencies.

eventType The value outOfCoverage indicates the UE to perform logging of measurements when the UE enters any cell selection state, and the value eventL1 indicates the UE to perform logging of measurements when the triggering condition (similar as event A2 as specified in 5.5.4.3) as configured in the event is met for the camping cell in camped normally state.

plmn-IdentityList Indicates a set of PLMNs defining when the UE performs measurement logging as well as the associated status indication and information retrieval i.e. the UE performs these actions when the RPLMN is part of this set of PLMNs.

tce-Id Parameter Trace Collection Entity Id: See TS 32.422 [52].

traceRecordingSessionRef Parameter Trace Recording Session Reference: See TS 32.422 [52].

reportType Parameter configures the type of MDT configuration, specifically Periodic MDT conifguraiton or Event Triggerd MDT configuration.

8.1.6.1.2.1.3	Test description

8.1.6.1.2.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 and NR Cell 11.

-	System information combination NR-2 as defined in TS 38.508-1[4] clause 4.4.3.1.2 is used in NR cells.

UE:

-	None.

Preamble:

-	The UE is in state 3N-A on NR Cell 1 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

8.1.6.1.2.1.3.2	Test procedure sequence

Table 8.1.6.1.2.1.3.2-1/2 illustrate the downlink power levels and other changing parameters to be applied for the cell at various time instants of the test execution. The configuration "T0" indicates the initial conditions. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.6.1.2.1.3.2-1: Time instances of cell power level for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 11 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -78 | -85 | NR Cell 1 becomes the highest ranked cell. |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -85 | -78 | NR Cell 11 becomes the highest ranked cell. |

Table 8.1.6.1.2.1.3.2-2: Time instances of cell power level for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 11 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -91 | NR Cell 1 becomes the highest ranked cell. |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -91 | -82 | NR Cell 11 becomes the highest ranked cell. |

Table 8.1.6.1.2.1.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits a LoggedMeasurementConfiguration message to configure the UE to perform logging of measurement results while in RRC_IDLE. | <-- | NR RRC: LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| 3 | Wait 5s to allow UE to activate logging. | - | - | - | - |
| 4 | The SS changes NR Cell 1 and NR Cell 11 levels according to the row "T1" in table 8.1.6.1.2.1.3.2-1/2. (Note 1) | - | - | - | - |
| 5 | The UE transmits an RRCSetupRequest message on NR Cell 11 to initiate a tracking area update procedure. | --> | NR RRC: RRCSetupRequest | - | - |
| 6 | SS transmits an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 7 | Check: Does the UE include the IE logMeasAvailable in the RRCSetupComplete message? | --> | NR RRC: RRCSetupComplete 5GMM: REGISTRATION REQUEST | 2 | P |
| 8-10 | Steps 4 to 6a1 of generic test procedure in TS 38.508-1[4] Table 4.9.5.2.2-1 are performed on NR Cell 11. NOTE: The UE performs a REGISTRATION procedure update the registration of the actual tracking area and the RRC connection is released. | - | - | - | - |
| 11-18 | Steps 1 to 8 of the generic radio bearer establishment procedure in TS 38.508-1 [4], Table4.5.4.2-3 are executed to successfully complete the service request procedure on NR Cell 11. | - | - | - | - |
| 19 | The SS transmits a UEInformationRequest message on NR Cell 11. | <-- | NR RRC: UEInformationRequest | - | - |
| 20 | Check: Does the UE transmit a UEInformationResponse message with the IEs absoluteTimeStamp, traceReference, traceRecordingSessionRef and tce-Id set to same value as received in the LoggedMeasurementConfiguration message in step 1; and a LogMeasInfoList with at least two entries with serving cell idle mode measurements and where in at least one of the entries the IE measResultListNR include a neighbouring cell measurement of NR Cell 11; and where the relativeTimeStamp is increased between the subsequent LogMeasInfoList entries by at least the value of configured loggingInterval in the received LoggedMeasurementConfiguration message in step 1. | --> | NR RRC: UEInformationResponse | 1, 3, 4 | P |
| Note 1: The change of power levels is to trigger a cell re-selection procedure to make sure that the UE is logging neighbouring cell measurements (logging interval=2.56 seconds) of Cell NR 11 while t-Reselection timer is running (7 seconds). |  |  |  |  |  |

8.1.6.1.2.1.3.3	Specific message contents

Table 8.1.6.1.2.1.3.3-1: SIB2 for NR cell 1 (preamble)

| Derivation Path: TS 38.508-1 [4], table 4.6.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| t-ReselectionNR | 7 | seconds |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.2.1.3.3-2: LoggedMeasurementConfiguration (step 1, Table 8.1.6.1.2.1.3.2-3)

Derivation path: TS 38.508-1 [4], table 4.6.1-5AA with condition PERIODICAL

Table 8.1.6.1.2.1.3.3-3: RRCSetupComplete (steps 7 and 14, Table 8.1.6.1.2.1.3.2-3)

| Derivation path: TS 38.508-1 [4], table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ue-MeasurementsAvailable-r16 | UE-MeasurementsAvailable-r16 with condition LOG |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.2.1.3.3-4: UEInformationRequest (step 19, Table 8.1.6.1.2.1.3.2-3)

Derivation path: TS 38.508-1 [4], table 4.6.1-32A with condition LOG

Table 8.1.6.1.2.1.3.3-5: UEInformationResponse (step 20, Table 8.1.6.1.2.1.3.2-3)

| Derivation path: TS 38.508-1 [4], table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| logMeasReport-r16 SEQUENCE { |  |  |  |
| absoluteTimeStamp-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceReference-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 SEQUENCE { |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |
| traceId-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |
| traceRecordingSessionRef-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| tce-Id-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r16 SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-r16 SEQUENCE { | At least 2 entries where at least one entry complies to entry with index ‘x’ below. SS records the relativeTimeStamp value for each entry |  |  |
| LogMeasInfo-r16[x] SEQUENCE { |  | entry x |  |
| locationInfo-r16 | Not checked |  |  |
| relativeTimeStamp-r16 | SS record the value |  |  |
| servCellIdentity-r16 | Same as NR Cell 1 |  |  |
| measResultServCell-16 SEQUENCE { |  |  |  |
| resultsSSB-Cell-r16 | MeasQuantityResults of NR Cell 1 |  |  |
| resultsSSB SEQUENCE { |  |  |  |
| best-ssb-Index | Not checked |  |  |
| best-ssb-Results | Not checked |  |  |
| numberOfGoodSSB | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells-r16 SEQUENCE { |  |  |  |
| measResultNeighCellListNR SEQUENCE (SIZE (1..maxFreq)) OF MeasResultLogging2NR-r16 SEQUENCE { | 1 entry |  |  |
| carrierFreq-r16 | Same as NR Cell 11 |  |  |
| measResultListLoggingNR -r16 SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultLoggingNR-r16 SEQUENCE { | 1 entry |  |  |
| MeasResultLoggingNR-r16[1] SEQUENCE { |  | entry 1 |  |
| physCellId-r16 | Same as NR Cell 11 |  |  |
| resultsSSB-Cell-r16 | MeasQuantityResults of NR Cell 11 |  |  |
| numberOfGoodSSB-r16 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCellListEUTRA | Not present |  |  |
| } |  |  |  |
| anyCellSelectionDetected-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| logMeasAvailable-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

###### 8.1.6.1.2.2	Logged MDT / RRC_INACTIVE / Logging and reporting / Inter-frequency measurement

8.1.6.1.2.2.1	Test Purpose (TP)

(1)

with { UE in NR RRC_INACTIVE state camping normally on an NR cell where logged measurement is configured without areaConfiguration and the UE is able to detect an NR inter-frequency cell }

ensure that {

when { T330 is running }

then { UE is logging serving cell measurements and Inter-frequency neighbouring cell measurements }

}

(2)

with { UE in NR RRC_INACTIVE state and has sent RRCResumeRequest message and UE has one or more logged Inter-frequency neighbouring cell measurement entries stored in VarLogMeasReport }

ensure that {

when { receiving RRCResume message }

then { UE includes the logMeasAvailable IE in the RRCResumeComplete message }

}

(3)

with { UE in NR RRC_CONNECTED state and UE has one or more logged Inter-frequency neighbouring cell measurement entries stored in VarLogMeasReport }

ensure that {

when { receiving UEInformationRequest message }

then { UE transmits UEInformationResponse messages with a logMeasReport with Inter-frequency neighbouring cell measurements }

}

(4)

with { UE in NR RRC_CONNECTED state and UE has one or more logged Inter-frequency neighbouring cell measurement entries stored in VarLogMeasReport }

ensure that {

when { receiving UEInformationRequest message and the UE has not transferred the total log in the UEInformationResponse message }

then { UE transmits UEInformationResponse messages with the logMeasAvailable IE }

}

8.1.6.1.2.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clauses 5.3.13.4, 5 5.5a.1.3, 5.5a.3.2, 5.7.10.3 and 6.2.2. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.3.13.4]

The UE shall:

1>	stop timer T319;

1>	stop timer T380, if running;

…

1>	set the content of the of RRCResumeComplete message as follows:

2>	if the upper layer provides NAS PDU, set the dedicatedNAS-Message to include the information received from upper layers;

…

2>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable in the RRCResumeComplete message;

2>	if the UE has Bluetooth logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailableBT in the RRCResumeComplete message;

2>	if the UE has WLAN logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailableWLAN in the RRCResumeComplete message;

…

[TS 38.331, clause 5.5a.1.3]

Upon receiving the LoggedMeasurementConfiguration message the UE shall:

1>	discard the logged measurement configuration as well as the logged measurement information as specified in 5.5a.2;

1>	store the received loggingDuration, reportType and areaConfiguration, if included, in VarLogMeasConfig;

1>	if the LoggedMeasurementConfiguration message includes plmn-IdentityList:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN as well as the PLMNs included in plmn-IdentityList;

1>	else:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN;

1>	store the received absoluteTimeInfo, traceReference, traceRecordingSessionRef, and tce-Id in VarLogMeasReport;

1>	store the received bt-NameList, if included, in VarLogMeasConfig;

1>	store the received wlan-Namelist, if included, in VarLogMeasConfig;

1>	store the received sensor-Namelist, if included, in VarLogMeasConfig;

1>	start timer T330 with the timer value set to the loggingDuration;

[TS 38.331, clause5.5a.3.2]

While T330 is running, the UE shall:

1>	perform the logging in accordance with the following:

2>	if the reportType is set to periodical in the VarLogMeasConfig:

3>	if the UE is camping normally on an NR cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and, if the cell is part of the area indicated by areaConfiguration if configured in VarLogMeasConfig:

4>	perform the logging at regular time intervals, as defined by the loggingInterval in the VarLogMeasConfig;

2>	else if the reportType is set to eventTriggered, and eventType is set to outOfCoverage:

3>	perform the logging at regular time intervals as defined by the loggingInterval in VarLogMeasConfig only when the UE is in any cell selection state;

3>	perform the logging immediately upon transitioning from the any cell selection state to the camped normally state;

2>	else if the reportType is set to eventTriggered and eventType is set to eventL1:

3>	if the UE is camping normally on an NR cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and, if the cell is part of the area indicated by areaConfiguration if configured in VarLogMeasConfig;

4>	perform the logging at regular time intervals as defined by the loggingInterval in VarLogMeasConfig only when the conditions indicated by the eventL1 are met;

2>	when performing the logging:

3>	set the relativeTimeStamp to indicate the elapsed time since the moment at which the logged measurement configuration was received;

3>	if detailed location information became available during the last logging interval, set the content of the locationInfo as in 5.3.3.7:

3>	if the UE is in any cell selection state (as specified in TS 38.304 [20]):

4>	set anyCellSelectionDetected to indicate the detection of no suitable or no acceptable cell found;

4>	set the servCellIdentity to indicate global cell identity of the last logged cell that the UE was camping on;

4>	set the measResultServingCell to include the quantities of the last logged cell the UE was camping on;

3>	else:

4>	set the servCellIdentity to indicate global cell identity of the cell the UE is camping on;

4>	set the measResultServingCell to include the quantities of the cell the UE is camping on;

4>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements that became available during the last logging interval for at most the following number of neighbouring cells: 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies per RAT and according to the following:

5>	for each neighbour cell included, include the optional fields that are available;

NOTE:	The UE includes the latest results of the available measurements as used for cell reselection evaluation in RRC_IDLE or RRC_INACTIVE, which are performed in accordance with the performance requirements as specified in TS 38.133 [14].

2>	when the memory reserved for the logged measurement information becomes full, stop timer T330 and perform the same actions as performed upon expiry of T330, as specified in 5.5a.1.4.

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if the logMeasReportReq is present and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3>	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3>	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3>	include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3>	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3>	include the logMeasInfoList and set it to include one or more entries from VarLogMeasReport starting from the entries logged first;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

3>	if the VarLogMeasReport includes one or more additional logged Bluetooth measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailableBT;

3>	if the VarLogMeasReport includes one or more additional logged WLAN measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailableWLAN;

…

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1.

[TS 38.331, clause 6.2.2]

–	LoggedMeasurementConfiguration

The LoggedMeasurementConfiguration message is used to perform logging of measurement results while in RRC_IDLE or RRC_INACTIVE. It is used to transfer the logged measurement configuration for network performance optimisation.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

LoggedMeasurementConfiguration message

-- ASN1START

-- TAG-LOGGEDMEASUREMENTCONFIGURATION-START

LoggedMeasurementConfiguration-r16 ::=  SEQUENCE {

criticalExtensions                      CHOICE {

loggedMeasurementConfiguration-r16      LoggedMeasurementConfiguration-r16-IEs,

criticalExtensionsFuture                SEQUENCE {}

}

}

LoggedMeasurementConfiguration-r16-IEs ::=  SEQUENCE {

traceReference-r16                          TraceReference-r16,

traceRecordingSessionRef-r16                OCTET STRING (SIZE (2)),

tce-Id-r16                                  OCTET STRING (SIZE (1)),

absoluteTimeInfo-r16                        AbsoluteTimeInfo-r16,

areaConfiguration-r16                       AreaConfiguration-r16                    OPTIONAL,  --Need R

plmn-IdentityList-r16                       PLMN-IdentityList2-r16                   OPTIONAL,  --Need R

bt-NameList-r16                             SetupRelease {BT-NameList-r16}           OPTIONAL,  --Need M

wlan-NameList-r16                           SetupRelease {WLAN-NameList-r16}         OPTIONAL,  --Need M

sensor-NameList-r16                         SetupRelease {Sensor-NameList-r16}       OPTIONAL,  --Need M

loggingDuration-r16                         LoggingDuration-r16,

reportType                                  CHOICE {

periodical                                  LoggedPeriodicalReportConfig-r16,

eventTriggered                              LoggedEventTriggerConfig-r16,

...

},

lateNonCriticalExtension                    OCTET STRING                             OPTIONAL,

nonCriticalExtension                        SEQUENCE {}                              OPTIONAL

}

LoggedPeriodicalReportConfig-r16 ::=            SEQUENCE {

loggingInterval-r16                             LoggingInterval-r16,

...

}

LoggedEventTriggerConfig-r16 ::=                SEQUENCE {

eventType-r16                                   EventType-r16,

loggingInterval-r16                             LoggingInterval-r16,

...

}

EventType-r16 ::= CHOICE {

outOfCoverage     NULL,

eventL1           SEQUENCE {

l1-Threshold      MeasTriggerQuantity,

hysteresis        Hysteresis,

timeToTrigger     TimeToTrigger

},

...

}

-- TAG-LOGGEDMEASUREMENTCONFIGURATION-STOP

-- ASN1STOP

LoggedMeasurementConfiguration field descriptions

absoluteTimeInfo Indicates the absolute time in the current cell.

areaConfiguration Used to restrict the area in which the UE performs measurement logging to cells broadcasting either one of the included cell identities or one of the included tracking area codes/ frequencies.

eventType The value outOfCoverage indicates the UE to perform logging of measurements when the UE enters any cell selection state, and the value eventL1 indicates the UE to perform logging of measurements when the triggering condition (similar as event A2 as specified in 5.5.4.3) as configured in the event is met for the camping cell in camped normally state.

plmn-IdentityList Indicates a set of PLMNs defining when the UE performs measurement logging as well as the associated status indication and information retrieval i.e. the UE performs these actions when the RPLMN is part of this set of PLMNs.

tce-Id Parameter Trace Collection Entity Id: See TS 32.422 [52].

traceRecordingSessionRef Parameter Trace Recording Session Reference: See TS 32.422 [52].

reportType Parameter configures the type of MDT configuration, specifically Periodic MDT conifguraiton or Event Triggerd MDT configuration.

8.1.6.1.2.2.3	Test description

8.1.6.1.2.2.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the serving cell and NR Cell 3 is a suitable neighbour inter-frequency cell.

-	System information combination NR-4 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

-	None.

Preamble:

-	The UE is in state 3N-A on NR Cell 1 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

8.1.6.1.2.2.3.2	Test procedure sequence

Table 8.1.6.1.2.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits a LoggedMeasurementConfiguration message with loggingInterval is set to ’320ms’ to configure the UE to perform logging of measurement results while in RRC_INACTIVE. | <-- | NR RRC: LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits an RRCRelease message. | <-- | NR RRC:RRCRelease | - | - |
| 3 | Wait 5 minutes to allow UE to activate logging. Note: Make the UE to log more than 520 entires of LogMeasInfo, and UE can’t transfer the total log in one UEInformationResponse message. | - | - | - | - |
| 4 | The SS transmits a Paging message including a matched identity (correct fullI-RNTI). | <-- | NR RRC: Paging | - | - |
| 5 | The UE transmits an RRCResumeRequest message by setting resumeIdentity to the stored shortI-RNTI value. | --> | NR RRC: RRCResumeRequest | - | - |
| 6 | The SS transmits an RRCResume message. | <-- | NR RRC: RRCResume | - | - |
| 7 | Check: Does the UE include the IE logMeasAvailable in the RRCResumeComplete message? | --> | NR RRC: RRCResumeComplete | 2 | P |
| 8 | The SS transmits a UEInformationRequest message on NR Cell 1. | <-- | NR RRC: UEInformationRequest | - | - |
| 9 | Check: Does the UE transmit a UEInformationResponse message with a LogMeasInfoList with at least one neighbouring cell measurement of NR Cell 3 and the logMeasAvailable IE? | --> | NR RRC: UEInformationResponse | 1, 3, 4 | P |
| 10 | The SS sends an UEInformationRequest message to get logMeasReport. | <-- | NR RRC: UEInformationRequest | - | - |
| 11 | The UE sends an UEInformationResponse message including logMeasReport. | --> | NR RRC: UEInformationResponse | - | - |

8.1.6.1.2.2.3.3	Specific message contents

Table 8.1.6.1.2.2.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.1.6.1.2.2.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-5AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| loggedMeasurementConfiguration-r16 SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| loggingInterval-r16 | ms320 | 0.32 seconds |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.2.2.3.3-2: RRCRelease (step 2, Table 8.1.6.1.2.2.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-16 with condition NR_RRC_INACTIVE

Table 8.1.6.1.2.2.3.3-3: RRCResumeComplete (step 7, Table 8.1.6.1.2.2.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCResumeComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcResumeComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ue-MeasurementsAvailable-r16 | UE-MeasurementsAvailable-r16 with condition LOG |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.2.2.3.3-4: UEInformationRequest (steps 8 and 10, Table 8.1.6.1.2.2.3.2-1)

Derivation path: TS 38.508-1 [4], Table 4.6.1-32A with condition LOG

Table 8.1.6.1.2.2.3.3-5: UEInformationResponse (steps 9 and 11, Table 8.1.6.1.2.2.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| logMeasReport-r16 SEQUENCE { |  |  |  |
| absoluteTimeStamp-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceReference-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 SEQUENCE { |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |
| traceId-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |
| traceRecordingSessionRef-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| tce-Id-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r16 SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-r16 SEQUENCE { | All entries comply to entry with index ‘x’ below. |  |  |
| LogMeasInfo-r16[x] SEQUENCE { |  | entry x |  |
| locationInfo-r16 | Not checked |  |  |
| relativeTimeStamp-r16 | SS record the value |  |  |
| servCellIdentity-r16 | Same as NR Cell 1 |  |  |
| measResultServCell-16 SEQUENCE { |  |  |  |
| resultsSSB-Cell-r16 | MeasQuantityResults of NR Cell 1 |  |  |
| resultsSSB SEQUENCE { |  |  |  |
| best-ssb-Index | Not checked |  |  |
| best-ssb-Results | Not checked |  |  |
| numberOfGoodSSB | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells-r16 SEQUENCE { |  |  |  |
| measResultNeighCellListNR SEQUENCE (SIZE (1..maxFreq)) OF MeasResultLogging2NR-r16 SEQUENCE { | 1 entry |  |  |
| MeasResultLogging2NR-r16[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r16 | Same as NR Cell 3 |  |  |
| measResultListLoggingNR -r16 SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultLoggingNR-r16 SEQUENCE { | 1 entry |  |  |
| MeasResultLoggingNR-r16[1] SEQUENCE { |  | entry 1 |  |
| physCellId-r16 | Same as NR Cell 3 |  |  |
| resultsSSB-Cell-r16 | MeasQuantityResults of NR Cell 3 |  |  |
| numberOfGoodSSB-r16 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCellListEUTRA | Not present |  |  |
| } |  |  |  |
| anyCellSelectionDetected-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| logMeasAvailable-r16 | True |  | Step 9 |
|  | Not checked |  | Step 11 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

###### 8.1.6.1.2.3	Logged MDT / Intra-frequency measurement, logging and reporting

8.1.6.1.2.3.1	Test Purpose (TP)

(1)

with { UE received LoggedMeasurementConfiguration message with a cellGlobalIdList on a NR cell }

ensure that {

when { UE camps on a NR cell in the cellGlobalIdList }

then { UE performs logged measurements }

}

(2)

with { UE received LoggedMeasurementConfiguration message with a cellGlobalIdList on a NR cell }

ensure that {
  when { UE camps on a NR cell not in the cellGlobalIdList }

then { UE does not perform logged measurements }

}

(3)

with { UE received LoggedMeasurementConfiguration message with a trackingAreaCodeList on a NR cell }

ensure that {
  when { UE camps on a NR cell in the trackingAreaCodeList }

then { UE performs logged measurements }

}

(4)

with { UE received LoggedMeasurementConfiguration message with a trackingAreaCodeList on a NR cell }

ensure that {
  when { UE camps on a NR cell not in the trackingAreaCodeList }

then { UE does not perform logged measurements }

}

8.1.6.1.2.3.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clauses 5.3.3.4, 5 5.5a.1.3, 5.5a.3.2, and 5.7.10.3. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.3.3.4]

The UE shall perform the following actions upon reception of the RRCSetup:

…

2>	set the content of RRCSetupComplete message as follows:

…

2>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable in the RRCSetupComplete message;

2>	if the UE has Bluetooth logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailableBT in the RRCSetupComplete message;

2>	if the UE has WLAN logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailableWLAN in the RRCSetupComplete message;

…

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if the logMeasReportReq is present and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3>	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3>	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3>	include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3>	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3>	include the logMeasInfoList and set it to include one or more entries from VarLogMeasReport starting from the entries logged first;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

3>	if the VarLogMeasReport includes one or more additional logged Bluetooth measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailableBT;

3>	if the VarLogMeasReport includes one or more additional logged WLAN measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailableWLAN;

…

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1.

[TS 38.331, clause 5.5a.1.3]

Upon receiving the LoggedMeasurementConfiguration message the UE shall:

1>	discard the logged measurement configuration as well as the logged measurement information as specified in 5.5a.2;

1>	store the received loggingDuration, reportType and areaConfiguration, if included, in VarLogMeasConfig;

1>	if the LoggedMeasurementConfiguration message includes plmn-IdentityList:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN as well as the PLMNs included in plmn-IdentityList;

1>	else:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN;

1>	store the received absoluteTimeInfo, traceReference, traceRecordingSessionRef, and tce-Id in VarLogMeasReport;

1>	store the received bt-NameList, if included, in VarLogMeasConfig;

1>	store the received wlan-Namelist, if included, in VarLogMeasConfig;

1>	store the received sensor-Namelist, if included, in VarLogMeasConfig;

1>	start timer T330 with the timer value set to the loggingDuration;

[TS 38.331, clause5.5a.3.2]

While T330 is running, the UE shall:

1>	perform the logging in accordance with the following:

2>	if the reportType is set to periodical in the VarLogMeasConfig:

3>	if the UE is camping normally on an NR cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and, if the cell is part of the area indicated by areaConfiguration if configured in VarLogMeasConfig:

4>	perform the logging at regular time intervals, as defined by the loggingInterval in the VarLogMeasConfig;

2>	else if the reportType is set to eventTriggered, and eventType is set to outOfCoverage:

3>	perform the logging at regular time intervals as defined by the loggingInterval in VarLogMeasConfig only when the UE is in any cell selection state;

3>	perform the logging immediately upon transitioning from the any cell selection state to the camped normally state;

2>	else if the reportType is set to eventTriggered and eventType is set to eventL1:

3>	if the UE is camping normally on an NR cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and, if the cell is part of the area indicated by areaConfiguration if configured in VarLogMeasConfig;

4>	perform the logging at regular time intervals as defined by the loggingInterval in VarLogMeasConfig only when the conditions indicated by the eventL1 are met;

2>	when performing the logging:

3>	set the relativeTimeStamp to indicate the elapsed time since the moment at which the logged measurement configuration was received;

3>	if detailed location information became available during the last logging interval, set the content of the locationInfo as in 5.3.3.7:

3>	if the UE is in any cell selection state (as specified in TS 38.304 [20]):

4>	set anyCellSelectionDetected to indicate the detection of no suitable or no acceptable cell found;

4>	set the servCellIdentity to indicate global cell identity of the last logged cell that the UE was camping on;

4>	set the measResultServingCell to include the quantities of the last logged cell the UE was camping on;

3>	else:

4>	set the servCellIdentity to indicate global cell identity of the cell the UE is camping on;

4>	set the measResultServingCell to include the quantities of the cell the UE is camping on;

4>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements that became available during the last logging interval for at most the following number of neighbouring cells: 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies per RAT and according to the following:

5>	for each neighbour cell included, include the optional fields that are available;

NOTE:	The UE includes the latest results of the available measurements as used for cell reselection evaluation in RRC_IDLE or RRC_INACTIVE, which are performed in accordance with the performance requirements as specified in TS 38.133 [14].

2>	when the memory reserved for the logged measurement information becomes full, stop timer T330 and 8.1.6.1.2.3.3	Test description

8.1.6.1.2.3.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1, NR Cell 2 and NR Cell 11.

-	System information combination NR-2 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

-	None.

Preamble:

-	The UE is in state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3 on NR Cell 1.

8.1.6.1.2.3.3.2	Test procedure sequence

Table 8.1.6.1.2.3.3.2-1/2 illustrate the downlink power levels and other changing parameters to be applied for the cell at various time instants of the test execution. The configuration "T0" indicates the initial conditions. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.6.1.2.3.3.2-1: Time instances of cell power level for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 11 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -78 | "Off" | "Off" | NR Cell 1 becomes the highest ranked cell. |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -85 | "Off" | -78 | NR Cell 11 becomes the highest ranked cell. |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -85 | -78 | "Off" | NR Cell 2 becomes the highest ranked cell. |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | -85 | "Off" | -78 | NR Cell 11 becomes the highest ranked cell. |
| T4 | SS/PBCH SSS EPRE | dBm/SCS | -78 | "Off" | -85 | NR Cell 1 becomes the highest ranked cell. |
| Note: The downlink signal level uncertainty is specified in TS 38.508-1 [4] clause 6.2.2.1. |  |  |  |  |  |  |

Table 8.1.6.1.2.3.3.2-2: Time instances of cell power level for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 11 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -82 | "Off" | "Off" | NR Cell 1 becomes the highest ranked cell. |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -91 | "Off" | -82 | NR Cell 11 becomes the highest ranked cell. |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -91 | -82 | "Off" | NR Cell 2 becomes the highest ranked cell. |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | -91 | "Off" | -82 | NR Cell 11 becomes the highest ranked cell. |
| T4 | SS/PBCH SSS EPRE | dBm/SCS | -82 | "Off" | -91 | NR Cell 1 becomes the highest ranked cell. |
| Note: The downlink signal level uncertainty is specified in TS 38.508-1 [4] clause 6.2.2.2.2. |  |  |  |  |  |  |

Table 8.1.6.1.2.3.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a LoggedMeasurementConfiguration message with a cellGlobalIdList on NR Cell 1. | <-- | NR RRC: LoggedMeasurementConfiguration | - | - |
| 2 | SS transmits a RRCRelease message to release the RRC connection. | <-- | NR RRC: RRCRelease | - | - |
| 3 | The SS changes NR Cell 1, NR Cell 2 and NR Cell 11 level according to the row "T1" in Table 8.1.6.1.2.3.3.2-1/2. | - | - | - | - |
| 4-9 | Steps 1 to 6a1 of generic test procedure in TS 38.508-1 [4] subclause 4.9.5.2.2-1 are performed on NR Cell 11. | - | - | - | - |
| 10 | Wait 30 seconds for UE logging interval timer to expire at least once. | - | - | - | - |
| 11-18 | Steps 1 to 8 of the generic radio bearer establishment procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure on NR Cell 11. | - | - | - | - |
| 19 | Void | - | - | - | - |
| 20 | The SS sends a UEInformationRequest message to get logMeasReport. | <-- | NR RRC: UEInformationRequest | - | - |
| 21 | Check: Does the UE send a UEInformationResponse message including at least one logMeasReport with serving cell measurements for NR Cell 11? | --> | NR RRC: UEInformationResponse | 1 | P |
| 22 | SS transmits a RRCRelease message to release the RRC connection. | <-- | NR RRC: RRCRelease | - | - |
| 23 | The SS changes NR Cell 1, NR Cell 2 and NR Cell 11 level according to the row "T2" in Table 8.1.6.1.2.3.3.2-1/2. | - | - | - | - |
| 24-29 | Steps 1 to 6a1 of generic test procedure in TS 38.508-1 [4] subclause 4.9.5.2.2-1 are performed on NR Cell 2. | - | - | - | - |
| 30 | Wait 30 seconds for UE logging interval timer to expire at least once. | - | - | - | - |
| 31 | The SS transmits a Paging message. | <-- | NR RRC: Paging | - | - |
| 32 | The UE transmits an RRCSetupRequest message. | --> | NR RRC: RRCSetupRequest | - | - |
| 33 | The SS transmits an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| - | EXCEPTION: In case the UE had performed a logging before the cell re-selection to NR Cell 2 then the steps 34a1-34a7 are executed. | - | - | - | - |
| 34a1 | The UE transmits an RRCSetupComplete message including logMeasAvailable IE set it to true. | --> | NR RRC: RRCSetupComplete | - | - |
| 34a2-34a5 | Steps 5 to 8 of the generic procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 34a6 | The SS sends a UEInformationRequest message to get logMeasReport. | <-- | NR RRC: UEInformationRequest | - | - |
| 34a7 | Check: Does the UE send a UEInformationResponse message with logged serving cell measurements of NR Cell 2? | --> | NR RRC: UEInformationResponse | 2 | F |
| - | EXCEPTION: In case the UE had not performed any logging before the cell re-selection to NR Cell 2 then the step 34b1 is executed. | - | - | - | - |
| 34b1 | Check: Does UE transmit an RRCSetupComplete message with logMeasAvailable IE not present? | --> | NR RRC: RRCSetupComplete | 2 | P |
| 34b2-34b5 | Steps 5 to 8 of the generic procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure. | - | - |  |  |
| 35 | The SS transmits a LoggedMeasurementConfiguration message with a trackingAreaCodeList on NR Cell 2. | <-- | NR RRC: LoggedMeasurementConfiguration | - | - |
| 36 | The SS transmits an RRCRelease message to release RRC connection and move to RRC_IDLE on NR Cell 2. | <-- | NR RRC: RRCRelease | - | - |
| 37 | The SS changes NR Cell 1, NR Cell 2 and NR Cell 11 level according to the row "T3" in Table 8.1.6.1.2.3.3.2-1/2. | - | - | - | - |
| 38-43 | Steps 1 to 6a1 of generic test procedure in TS 38.508-1 [4] subclause 4.9.5.2.2-1 are performed on NR Cell 11. | - | - | - | - |
| 44 | The SS transmits a Paging message on NR Cell 11. | <-- | NR RRC: Paging | - | - |
| 45 | The UE transmits an RRCSetupRequest message on NR Cell 11. | --> | NR RRC: RRCSetupRequest | - | - |
| 46 | The SS transmits an RRCSetup message on NR Cell 11. | <-- | NR RRC: RRCSetup | - | - |
| - | EXCEPTION: In case the UE had performed a logging before the cell re-selection to NR Cell 11 then the steps 47a1-47a7 are executed. | - | - | - | - |
| 47a1 | The UE transmits an RRCSetupComplete message including logMeasAvailable IE set it to true. | --> | NR RRC: RRCSetupComplete | - | - |
| 47a2-47a5 | Steps 5 to 8 of the generic procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 47a6 | The SS sends a UEInformationRequest message to get logMeasReport. | <-- | NR RRC: UEInformationRequest | - | - |
| 47a7 | Check: Does the UE send a UEInformationResponse message with logged serving cell measurements of NR Cell 11? | --> | NR RRC: UEInformationResponse | 4 | F |
| - | EXCEPTION: In case the UE had not performed any logging before the cell re-selection to NR Cell 11 then the step 47b1-47b5 are executed. | - | - | - | - |
| 47b1 | Check: Does UE transmit an RRCSetupComplete message with logMeasAvailable IE not present? | --> | NR RRC: RRCSetupComplete | 4 | P |
| 47b2-47b5 | Steps 5 to 8 of the generic procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 48 | The SS transmits an RRCRelease message to release the RRC connection. | <-- | NR RRC: RRCRelease | - | - |
| 49 | The SS changes NR Cell 1, NR Cell 2 and NR Cell 11 level according to the row "T4" in Table 8.1.6.1.2.3.3.2-1/2. | - | - | - | - |
| 50-55 | Steps 1 to 6a1 of generic test procedure in TS 38.508-1 [4] subclause 4.9.5.2.2-1 are performed on NR Cell 1. | - | - | - | - |
| 56 | Wait 30 seconds for UE logging interval timer to expire at least once. | - | - | - | - |
| 57-64 | Steps 1 to 8 of the generic procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 65 | Void | - | - | - | - |
| 66 | The SS sends a UEInformationRequest message to get logMeasReport. | <-- | NR RRC: UEInformationRequest | - | - |
| 67 | Check: Does the UE send a UEInformationResponse include logMeasReport with logged serving cell measurements of NR Cell 1? | --> | NR RRC: UEInformationResponse | 3 | P |

8.1.6.1.2.3.3.3	Specific message contents

Table 8.1.6.1.2.3.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.1.6.1.2.3.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-5AA with condition PERIODICAL. |
| --- |
| Information Element | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| loggedMeasurementConfiguration-r16 SEQUENCE { |  |  |  |
| areaConfiguration-r16 SEQUENCE { |  |  |  |
| areaConfig-r16 CHOICE { |  |  |  |
| CellGlobalIdList-r16 SEQUENCE (SIZE (1)) OF CGI-Info-Logging-r16 { |  |  |  |
| plmn-Identity-r16 | Set to the same Mobile Country Code and Mobile Network Code stored in EFIMSI on the test USIM card |  |  |
| cellIdentity-r16 | Same as NR Cell 11 |  |  |
| trackingAreaCode-r16 | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.2.3.3.3-2: LoggedMeasurementConfiguration (step 35, Table 8.1.6.1.2.3.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-5AA with condition PERIODICAL. |
| --- |
| Information Element | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| loggedMeasurementConfiguration-r16 SEQUENCE { |  |  |  |
| areaConfiguration-r16 SEQUENCE { |  |  |  |
| areaConfig-r16 CHOICE { |  |  |  |
| trackingAreaCodeList-r16 SEQUENCE (SIZE (1…)) OF TrackingAreaCode { | 1 entry |  |  |
| TrackingAreaCode | 1 | Entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.2.3.3.3-3: RRCSetupComplete (steps 34b1 and 47b1, Table 8.1.6.1.2.3.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-22. |
| --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { | Checked | Note 1 |  |
| ue-MeasurementsAvailable-r16 SEQUENCE { |  |  | If nonCriticalExtension is present |
| logMeasAvailable-r16 | Not present |  | If ue-MeasurementsAvailable-r16 is present |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| If nonCriticalExtension and ue-MeasurementsAvailable-r16 are present | Contents of ue-MeasurementsAvailable-r16 shall be checked only if nonCriticalExtension and ue-MeasurementsAvailable-r16 are present |
| Note 1: If the UE has no underlying information to report to the network, this information element may be absent. If this information element and MeasurementsAvailable-r16 are present, further check on the contents of the ue-MeasurementsAvailable-r16 shall be performed. |  |

Table 8.1.6.1.2.3.3.3-4: RRCSetupComplete (steps 14, 34a1, 47a1, and 60, Table 8.1.6.1.2.3.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ue-MeasurementsAvailable-r16 | UE-MeasurementsAvailable-r16 with condition LOG |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.2.3.3.3-5: UEInformationRequest (steps 20, 34a6, 47a6, and 66, Table 8.1.6.1.2.3.3.2-3)

Derivation path: TS 38.508-1 [4], Table 4.6.1-32A with condition LOG

Table 8.1.6.1.2.3.3.3-6: UEInformationResponse (steps 21 and 34a7, Table 8.1.6.1.2.3.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| logMeasReport-r16 SEQUENCE { |  |  |  |
| absoluteTimeStamp-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceReference-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 SEQUENCE { |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |
| traceId-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |
| traceRecordingSessionRef-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| tce-Id-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r16 SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-r16 SEQUENCE { | At least one entry and all entries complies to entry with index ‘x’ below. |  |  |
| LogMeasInfo-r16[x] SEQUENCE { |  | entry x |  |
| locationInfo-r16 | Not checked |  |  |
| relativeTimeStamp-r16 | SS record the value |  |  |
| servCellIdentity-r16 | Same as NR Cell 11 | No logged serving cell measurement of NR Cell 2 |  |
| measResultServCell-16 SEQUENCE { |  |  |  |
| resultsSSB-Cell-r16 | MeasQuantityResults of NR Cell 11 |  |  |
| resultsSSB SEQUENCE { |  |  |  |
| best-ssb-Index | Not checked |  |  |
| best-ssb-Results | Not checked |  |  |
| numberOfGoodSSB | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells-r16 | Any allowed value |  |  |
| anyCellSelectionDetected-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| logMeasAvailable-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.2.3.3.3-7: UEInformationResponse (step 47a7, Table 8.1.6.1.2.3.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| logMeasReport-r16 SEQUENCE { |  |  |  |
| absoluteTimeStamp-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 35 |  |  |
| traceReference-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 SEQUENCE { |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit | Same value as sent by SS in LoggedMeasurementConfiguration in step 35 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit | Same value as sent by SS in LoggedMeasurementConfiguration in step 35 |  |  |
| } |  |  |  |
| traceId-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 35 |  |  |
| } |  |  |  |
| traceRecordingSessionRef-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 35 |  |  |
| tce-Id-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r16 SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-r16 SEQUENCE { | At least one entry and all entries complies to entry with index ‘x’ below. |  |  |
| LogMeasInfo-r16[x] SEQUENCE { |  | entry x |  |
| locationInfo-r16 | Not checked |  |  |
| relativeTimeStamp-r16 | SS record the value |  |  |
| servCellIdentity-r16 | Same as NR Cell 2 | No logged serving cell measurement of NR Cell 11 |  |
| measResultServCell-16 SEQUENCE { |  |  |  |
| resultsSSB-Cell-r16 | MeasQuantityResults of NR Cell 2 |  |  |
| resultsSSB SEQUENCE { |  |  |  |
| best-ssb-Index | Not checked |  |  |
| best-ssb-Results | Not checked |  |  |
| numberOfGoodSSB | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells-r16 | Any allowed value |  |  |
| anyCellSelectionDetected-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| logMeasAvailable-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.2.3.3.3-8: UEInformationResponse (step 67, Table 8.1.6.1.2.3.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| logMeasReport-r16 SEQUENCE { |  |  |  |
| absoluteTimeStamp-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 35 |  |  |
| traceReference-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 SEQUENCE { |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit | Same value as sent by SS in LoggedMeasurementConfiguration in step 35 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit | Same value as sent by SS in LoggedMeasurementConfiguration in step 35 |  |  |
| } |  |  |  |
| traceId-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 35 |  |  |
| } |  |  |  |
| traceRecordingSessionRef-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 35 |  |  |
| tce-Id-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r16 SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-r16 SEQUENCE { | At least one entry complies to entry with index ‘x’ below. |  |  |
| LogMeasInfo-r16[x] SEQUENCE { |  | entry x |  |
| locationInfo-r16 | Not checked |  |  |
| relativeTimeStamp-r16 | SS record the value |  |  |
| servCellIdentity-r16 | Same as NR Cell 1 | No logged serving cell measurement of NR Cell 11 |  |
| measResultServCell-16 [x] SEQUENCE { |  |  |  |
| resultsSSB-Cell-r16 | MeasQuantityResults of NR Cell 1 |  |  |
| resultsSSB SEQUENCE { |  |  |  |
| best-ssb-Index | Not checked |  |  |
| best-ssb-Results | Not checked |  |  |
| numberOfGoodSSB | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells-r16 | Any allowed value |  |  |
| anyCellSelectionDetected-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| logMeasAvailable-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

###### 8.1.6.1.2.4	Logged MDT/ RRC_IDLE / Logging and reporting / periodic measurement trigger

8.1.6.1.2.4.1	Test Purpose (TP)

(1)

with { UE in NR RRC_IDLE state camping normally on an NR cell where reportType is set to periodical and the UE is able to detect an intra-frequency cell }

ensure that {

when { T330 is running }

then { UE is logging serving cell idle mode measurements and intra-frequency neighbouring cell measurements at regular time intervals }

}

8.1.6.1.2.4.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clauses 5.3.3.4, 5 5.5a.1.3, 5.5a.3.2, 5.7.10.3 and 6.2.2. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.3.3.4]

The UE shall perform the following actions upon reception of the RRCSetup:

…

3>	set the content of RRCSetupComplete message as follows:

…

2>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable in the RRCSetupComplete message;

2>	if the UE has Bluetooth logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailableBT in the RRCSetupComplete message;

2>	if the UE has WLAN logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailableWLAN in the RRCSetupComplete message;

…

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if the logMeasReportReq is present and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3>	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3>	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3>	include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3>	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3>	include the logMeasInfoList and set it to include one or more entries from VarLogMeasReport starting from the entries logged first;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

3>	if the VarLogMeasReport includes one or more additional logged Bluetooth measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailableBT;

3>	if the VarLogMeasReport includes one or more additional logged WLAN measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailableWLAN;

…

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1.

[TS 38.331, clause 5.5a.1.3]

Upon receiving the LoggedMeasurementConfiguration message the UE shall:

1>	discard the logged measurement configuration as well as the logged measurement information as specified in 5.5a.2;

1>	store the received loggingDuration, reportType and areaConfiguration, if included, in VarLogMeasConfig;

1>	if the LoggedMeasurementConfiguration message includes plmn-IdentityList:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN as well as the PLMNs included in plmn-IdentityList;

1>	else:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN;

1>	store the received absoluteTimeInfo, traceReference, traceRecordingSessionRef, and tce-Id in VarLogMeasReport;

1>	store the received bt-NameList, if included, in VarLogMeasConfig;

1>	store the received wlan-Namelist, if included, in VarLogMeasConfig;

1>	store the received sensor-Namelist, if included, in VarLogMeasConfig;

1>	start timer T330 with the timer value set to the loggingDuration;

[TS 38.331, clause5.5a.3.2]

While T330 is running, the UE shall:

1>	perform the logging in accordance with the following:

2>	if the reportType is set to periodical in the VarLogMeasConfig:

3>	if the UE is camping normally on an NR cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and, if the cell is part of the area indicated by areaConfiguration if configured in VarLogMeasConfig:

4>	perform the logging at regular time intervals, as defined by the loggingInterval in the VarLogMeasConfig;

2>	else if the reportType is set to eventTriggered, and eventType is set to outOfCoverage:

3>	perform the logging at regular time intervals as defined by the loggingInterval in VarLogMeasConfig only when the UE is in any cell selection state;

3>	perform the logging immediately upon transitioning from the any cell selection state to the camped normally state;

2>	else if the reportType is set to eventTriggered and eventType is set to eventL1:

3>	if the UE is camping normally on an NR cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and, if the cell is part of the area indicated by areaConfiguration if configured in VarLogMeasConfig;

4>	perform the logging at regular time intervals as defined by the loggingInterval in VarLogMeasConfig only when the conditions indicated by the eventL1 are met;

2>	when performing the logging:

3>	set the relativeTimeStamp to indicate the elapsed time since the moment at which the logged measurement configuration was received;

3>	if detailed location information became available during the last logging interval, set the content of the locationInfo as in 5.3.3.7:

3>	if the UE is in any cell selection state (as specified in TS 38.304 [20]):

4>	set anyCellSelectionDetected to indicate the detection of no suitable or no acceptable cell found;

4>	set the servCellIdentity to indicate global cell identity of the last logged cell that the UE was camping on;

4>	set the measResultServingCell to include the quantities of the last logged cell the UE was camping on;

3>	else:

4>	set the servCellIdentity to indicate global cell identity of the cell the UE is camping on;

4>	set the measResultServingCell to include the quantities of the cell the UE is camping on;

4>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements that became available during the last logging interval for at most the following number of neighbouring cells: 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies per RAT and according to the following:

5>	for each neighbour cell included, include the optional fields that are available;

NOTE:	The UE includes the latest results of the available measurements as used for cell reselection evaluation in RRC_IDLE or RRC_INACTIVE, which are performed in accordance with the performance requirements as specified in TS 38.133 [14].

2>	when the memory reserved for the logged measurement information becomes full, stop timer T330 and perform the same actions as performed upon expiry of T330, as specified in 5.5a.1.4.

[TS 38.331, clause 6.2.2]

–	LoggedMeasurementConfiguration

The LoggedMeasurementConfiguration message is used to perform logging of measurement results while in RRC_IDLE or RRC_INACTIVE. It is used to transfer the logged measurement configuration for network performance optimisation.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

LoggedMeasurementConfiguration message

-- ASN1START

-- TAG-LOGGEDMEASUREMENTCONFIGURATION-START

LoggedMeasurementConfiguration-r16 ::=  SEQUENCE {

criticalExtensions                      CHOICE {

loggedMeasurementConfiguration-r16      LoggedMeasurementConfiguration-r16-IEs,

criticalExtensionsFuture                SEQUENCE {}

}

}

LoggedMeasurementConfiguration-r16-IEs ::=  SEQUENCE {

traceReference-r16                          TraceReference-r16,

traceRecordingSessionRef-r16                OCTET STRING (SIZE (2)),

tce-Id-r16                                  OCTET STRING (SIZE (1)),

absoluteTimeInfo-r16                        AbsoluteTimeInfo-r16,

areaConfiguration-r16                       AreaConfiguration-r16                    OPTIONAL,  --Need R

plmn-IdentityList-r16                       PLMN-IdentityList2-r16                   OPTIONAL,  --Need R

bt-NameList-r16                             SetupRelease {BT-NameList-r16}           OPTIONAL,  --Need M

wlan-NameList-r16                           SetupRelease {WLAN-NameList-r16}         OPTIONAL,  --Need M

sensor-NameList-r16                         SetupRelease {Sensor-NameList-r16}       OPTIONAL,  --Need M

loggingDuration-r16                         LoggingDuration-r16,

reportType                                  CHOICE {

periodical                                  LoggedPeriodicalReportConfig-r16,

eventTriggered                              LoggedEventTriggerConfig-r16,

...

},

lateNonCriticalExtension                    OCTET STRING                             OPTIONAL,

nonCriticalExtension                        SEQUENCE {}                              OPTIONAL

}

LoggedPeriodicalReportConfig-r16 ::=            SEQUENCE {

loggingInterval-r16                             LoggingInterval-r16,

...

}

LoggedEventTriggerConfig-r16 ::=                SEQUENCE {

eventType-r16                                   EventType-r16,

loggingInterval-r16                             LoggingInterval-r16,

...

}

EventType-r16 ::= CHOICE {

outOfCoverage     NULL,

eventL1           SEQUENCE {

l1-Threshold      MeasTriggerQuantity,

hysteresis        Hysteresis,

timeToTrigger     TimeToTrigger

},

...

}

-- TAG-LOGGEDMEASUREMENTCONFIGURATION-STOP

-- ASN1STOP

LoggedMeasurementConfiguration field descriptions

absoluteTimeInfo Indicates the absolute time in the current cell.

areaConfiguration Used to restrict the area in which the UE performs measurement logging to cells broadcasting either one of the included cell identities or one of the included tracking area codes/ frequencies.

eventType The value outOfCoverage indicates the UE to perform logging of measurements when the UE enters any cell selection state, and the value eventL1 indicates the UE to perform logging of measurements when the triggering condition (similar as event A2 as specified in 5.5.4.3) as configured in the event is met for the camping cell in camped normally state.

plmn-IdentityList Indicates a set of PLMNs defining when the UE performs measurement logging as well as the associated status indication and information retrieval i.e. the UE performs these actions when the RPLMN is part of this set of PLMNs.

tce-Id Parameter Trace Collection Entity Id: See TS 32.422 [52].

traceRecordingSessionRef Parameter Trace Recording Session Reference: See TS 32.422 [52].

reportType Parameter configures the type of MDT configuration, specifically Periodic MDT conifguraiton or Event Triggerd MDT configuration.

8.1.6.1.2.4.3	Test description

8.1.6.1.2.4.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 and NR Cell 11.

-	System information combination NR-2 as defined in TS 38.508-1[4] clause 4.4.3.1.2 is used in NR cells.

UE:

-	None.

Preamble:

-	The UE is in state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3 on NR Cell 1.

8.1.6.1.2.4.3.2	Test procedure sequence

Table 8.1.6.1.2.4.3.2-1/2 illustrate the downlink power levels and other changing parameters to be applied for the cell at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.6.1.2.4.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 11 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -78 | -85 | NR Cell 1 becomes the highest ranked cell. |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -85 | -78 | NR Cell 11 becomes the highest ranked cell. |

Table 8.1.6.1.2.4.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 11 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -91 | NR Cell 1 becomes the highest ranked cell. |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -91 | -82 | NR Cell 11 becomes the highest ranked cell. |

Table 8.1.6.1.2.4.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits a LoggedMeasurementConfiguration message with reportType is set to ‘periodical’ and LoggingInterval is set to ‘ms10240’, to configure the UE to perform logging of measurement results while in RRC_IDLE. | <-- | NR RRC: LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| 3 | Wait 5s and the LoggingInterval will not be expired. | - | - | - | - |
| 4 | The SS changes NR Cell 1 and NR Cell 11 levels according to the row "T1" in table 8.1.6.1.2.4.3.2-1/2. | - | - | - | - |
| 5 | The UE transmits an RRCSetupRequest message on NR Cell 11 to initiate a tracking area update procedure. | --> | NR RRC: RRCSetupRequest | - | - |
| 6 | SS transmits an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 7 | Check: Does the UE include the IE logMeasAvailable in the RRCSetupComplete message? | --> | NR RRC: RRCSetupComplete 5GMM: REGISTRATION REQUEST | 1 | F |
| 8-10 | Steps 4 to 6a1 of generic test procedure in TS 38.508-1 [4] subclause 4.9.5.2.2-1 are performed on NR Cell 11. | - | - | - | - |
| 11 | Wait 30s to allow UE to activate logging. | - | - | - | - |
| 12-14 | Steps 1 to 3 of the generic procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 15 | Check: Does the UE include the IE logMeasAvailable in the RRCSetupComplete message? | --> | NR RRC: RRCSetupComplete 5GMM: SERVICE REQUEST | 1 | P |
| 16-19 | Steps 5 to 8 of the generic procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 20 | The SS transmits a UEInformationRequest message on NR Cell 11. | <-- | NR RRC: UEInformationRequest | - | - |
| 21 | Check: Does the UE transmit a UEInformationResponse message? | --> | NR RRC: UEInformationResponse | 1 | P |

8.1.6.1.2.4.3.3	Specific message contents

Table 8.1.6.1.2.4.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.1.6.1.2.4.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-5AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| loggedMeasurementConfiguration-r16 SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| loggingInterval-r16 | ms10240 | 10.24 seconds |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.2.4.3.3-2: Void

Table 8.1.6.1.2.4.3.3-3: RRCSetupComplete (steps 7 and 15, Table 8.1.6.1.2.4.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |
| nonCriticalExtension { |  |  |  |
| ue-MeasurementsAvailable-r16 | UE-MeasurementsAvailable-r16 with condition LOG |  | Step 15 |
| ue-MeasurementsAvailable-r16 | UE-logMeasurements-NotAvailable | Table 8.1.6.1.2.4.3.3-3A | Step 7 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.2.4.3.3-3A: UE-logMeasurements-NotAvailable (step 7, Table 8.1.6.1.2.5.3.2-3)

| Derivation path: TS 38.508-1[4], Table 4.6.5-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE-MeasurementsAvailable-r16 ::= SEQUENCE { |  |  |  |  |
| logMeasAvailable-r16 |  | Not Present |  |  |

Table 8.1.6.1.2.4.3.3-4: UEInformationRequest (step 20, Table 8.1.6.1.2.4.3.2-3)

Derivation path: TS 38.508-1 [4], Table 4.6.1-32A with condition LOG

Table 8.1.6.1.2.4.3.3-5: UEInformationResponse (step 21, Table 8.1.6.1.2.4.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| logMeasReport-r16 SEQUENCE { |  |  |  |
| absoluteTimeStamp-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceReference-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 SEQUENCE { |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |
| traceId-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |
| traceRecordingSessionRef-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| tce-Id-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r16 SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-r16 SEQUENCE { | At least 2 entries where at least one entry complies to entry with index ‘x’ below. SS records the relativeTimeStamp value for each entry |  |  |
| LogMeasInfo-r16[x] SEQUENCE { |  | entry x |  |
| locationInfo-r16 | Not checked |  |  |
| relativeTimeStamp-r16 | SS record the value |  |  |
| servCellIdentity-r16 | Same as NR Cell 11 |  |  |
| measResultServCell-16 SEQUENCE { |  |  |  |
| resultsSSB-Cell-r16 | MeasQuantityResults of NR Cell 11 |  |  |
| resultsSSB SEQUENCE { |  |  |  |
| best-ssb-Index | Not checked |  |  |
| best-ssb-Results | Not checked |  |  |
| numberOfGoodSSB | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells-r16 SEQUENCE { |  |  |  |
| measResultNeighCellListNR SEQUENCE (SIZE (1..maxFreq)) OF MeasResultLogging2NR-r16 SEQUENCE { | 1 entry |  |  |
| MeasResultLogging2NR-r16[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r16 | Same as NR Cell 1 |  |  |
| measResultListLoggingNR -r16 SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultLoggingNR-r16 SEQUENCE { | 1 entry |  |  |
| MeasResultLoggingNR-r16[1] SEQUENCE { |  | entry 1 |  |
| physCellId-r16 | Same as NR Cell 1 |  |  |
| resultsSSB-Cell-r16 | MeasQuantityResults of NR Cell 1 |  |  |
| numberOfGoodSSB-r16 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCellListEUTRA | Not present |  |  |
| } |  |  |  |
| anyCellSelectionDetected-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| logMeasAvailable-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

###### 8.1.6.1.2.5	Logged MDT/ RRC_IDLE / Logging and reporting / event-based trigger

8.1.6.1.2.5.1	Test Purpose (TP)

(1)

with { UE in NR RRC_IDLE state camping normally on an NR cell where reportType is set to eventTriggered and eventType is set to eventL1 }

ensure that {

when { T330 is running }

then { UE is logging serving cell idle mode measurements only when the conditions indicated by the eventL1 are met }

}

8.1.6.1.2.5.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clauses 5.3.3.4, 5 5.5a.1.3, 5.5a.3.2, 5.7.10.3 and 6.2.2. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.3.3.4]

The UE shall perform the following actions upon reception of the RRCSetup:

…

4>	set the content of RRCSetupComplete message as follows:

…

2>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable in the RRCSetupComplete message;

2>	if the UE has Bluetooth logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailableBT in the RRCSetupComplete message;

2>	if the UE has WLAN logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailableWLAN in the RRCSetupComplete message;

…

[TS 38.331, clause 5.5a.1.3]

Upon receiving the LoggedMeasurementConfiguration message the UE shall:

1>	discard the logged measurement configuration as well as the logged measurement information as specified in 5.5a.2;

1>	store the received loggingDuration, reportType and areaConfiguration, if included, in VarLogMeasConfig;

1>	if the LoggedMeasurementConfiguration message includes plmn-IdentityList:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN as well as the PLMNs included in plmn-IdentityList;

1>	else:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN;

1>	store the received absoluteTimeInfo, traceReference, traceRecordingSessionRef, and tce-Id in VarLogMeasReport;

1>	store the received bt-NameList, if included, in VarLogMeasConfig;

1>	store the received wlan-Namelist, if included, in VarLogMeasConfig;

1>	store the received sensor-Namelist, if included, in VarLogMeasConfig;

1>	start timer T330 with the timer value set to the loggingDuration;

[TS 38.331, clause5.5a.3.2]

While T330 is running, the UE shall:

1>	perform the logging in accordance with the following:

2>	if the reportType is set to periodical in the VarLogMeasConfig:

3>	if the UE is camping normally on an NR cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and, if the cell is part of the area indicated by areaConfiguration if configured in VarLogMeasConfig:

4>	perform the logging at regular time intervals, as defined by the loggingInterval in the VarLogMeasConfig;

2>	else if the reportType is set to eventTriggered, and eventType is set to outOfCoverage:

3>	perform the logging at regular time intervals as defined by the loggingInterval in VarLogMeasConfig only when the UE is in any cell selection state;

3>	perform the logging immediately upon transitioning from the any cell selection state to the camped normally state;

2>	else if the reportType is set to eventTriggered and eventType is set to eventL1:

3>	if the UE is camping normally on an NR cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and, if the cell is part of the area indicated by areaConfiguration if configured in VarLogMeasConfig;

4>	perform the logging at regular time intervals as defined by the loggingInterval in VarLogMeasConfig only when the conditions indicated by the eventL1 are met;

2>	when performing the logging:

3>	set the relativeTimeStamp to indicate the elapsed time since the moment at which the logged measurement configuration was received;

3>	if detailed location information became available during the last logging interval, set the content of the locationInfo as in 5.3.3.7:

3>	if the UE is in any cell selection state (as specified in TS 38.304 [20]):

4>	set anyCellSelectionDetected to indicate the detection of no suitable or no acceptable cell found;

4>	set the servCellIdentity to indicate global cell identity of the last logged cell that the UE was camping on;

4>	set the measResultServingCell to include the quantities of the last logged cell the UE was camping on;

3>	else:

4>	set the servCellIdentity to indicate global cell identity of the cell the UE is camping on;

4>	set the measResultServingCell to include the quantities of the cell the UE is camping on;

4>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements that became available during the last logging interval for at most the following number of neighbouring cells: 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies per RAT and according to the following:

5>	for each neighbour cell included, include the optional fields that are available;

NOTE:	The UE includes the latest results of the available measurements as used for cell reselection evaluation in RRC_IDLE or RRC_INACTIVE, which are performed in accordance with the performance requirements as specified in TS 38.133 [14].

2>	when the memory reserved for the logged measurement information becomes full, stop timer T330 and perform the same actions as performed upon expiry of T330, as specified in 5.5a.1.4.

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if the logMeasReportReq is present and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3>	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3>	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3>	include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3>	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3>	include the logMeasInfoList and set it to include one or more entries from VarLogMeasReport starting from the entries logged first;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

3>	if the VarLogMeasReport includes one or more additional logged Bluetooth measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailableBT;

3>	if the VarLogMeasReport includes one or more additional logged WLAN measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailableWLAN;

…

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1.

[TS 38.331, clause 6.2.2]

–	LoggedMeasurementConfiguration

The LoggedMeasurementConfiguration message is used to perform logging of measurement results while in RRC_IDLE or RRC_INACTIVE. It is used to transfer the logged measurement configuration for network performance optimisation.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

LoggedMeasurementConfiguration message

-- ASN1START

-- TAG-LOGGEDMEASUREMENTCONFIGURATION-START

LoggedMeasurementConfiguration-r16 ::=  SEQUENCE {

criticalExtensions                      CHOICE {

loggedMeasurementConfiguration-r16      LoggedMeasurementConfiguration-r16-IEs,

criticalExtensionsFuture                SEQUENCE {}

}

}

LoggedMeasurementConfiguration-r16-IEs ::=  SEQUENCE {

traceReference-r16                          TraceReference-r16,

traceRecordingSessionRef-r16                OCTET STRING (SIZE (2)),

tce-Id-r16                                  OCTET STRING (SIZE (1)),

absoluteTimeInfo-r16                        AbsoluteTimeInfo-r16,

areaConfiguration-r16                       AreaConfiguration-r16                    OPTIONAL,  --Need R

plmn-IdentityList-r16                       PLMN-IdentityList2-r16                   OPTIONAL,  --Need R

bt-NameList-r16                             SetupRelease {BT-NameList-r16}           OPTIONAL,  --Need M

wlan-NameList-r16                           SetupRelease {WLAN-NameList-r16}         OPTIONAL,  --Need M

sensor-NameList-r16                         SetupRelease {Sensor-NameList-r16}       OPTIONAL,  --Need M

loggingDuration-r16                         LoggingDuration-r16,

reportType                                  CHOICE {

periodical                                  LoggedPeriodicalReportConfig-r16,

eventTriggered                              LoggedEventTriggerConfig-r16,

...

},

lateNonCriticalExtension                    OCTET STRING                             OPTIONAL,

nonCriticalExtension                        SEQUENCE {}                              OPTIONAL

}

LoggedPeriodicalReportConfig-r16 ::=            SEQUENCE {

loggingInterval-r16                             LoggingInterval-r16,

...

}

LoggedEventTriggerConfig-r16 ::=                SEQUENCE {

eventType-r16                                   EventType-r16,

loggingInterval-r16                             LoggingInterval-r16,

...

}

EventType-r16 ::= CHOICE {

outOfCoverage     NULL,

eventL1           SEQUENCE {

l1-Threshold      MeasTriggerQuantity,

hysteresis        Hysteresis,

timeToTrigger     TimeToTrigger

},

...

}

-- TAG-LOGGEDMEASUREMENTCONFIGURATION-STOP

-- ASN1STOP

LoggedMeasurementConfiguration field descriptions

absoluteTimeInfo Indicates the absolute time in the current cell.

areaConfiguration Used to restrict the area in which the UE performs measurement logging to cells broadcasting either one of the included cell identities or one of the included tracking area codes/ frequencies.

eventType The value outOfCoverage indicates the UE to perform logging of measurements when the UE enters any cell selection state, and the value eventL1 indicates the UE to perform logging of measurements when the triggering condition (similar as event A2 as specified in 5.5.4.3) as configured in the event is met for the camping cell in camped normally state.

plmn-IdentityList Indicates a set of PLMNs defining when the UE performs measurement logging as well as the associated status indication and information retrieval i.e. the UE performs these actions when the RPLMN is part of this set of PLMNs.

tce-Id Parameter Trace Collection Entity Id: See TS 32.422 [52].

traceRecordingSessionRef Parameter Trace Recording Session Reference: See TS 32.422 [52].

reportType Parameter configures the type of MDT configuration, specifically Periodic MDT conifguraiton or Event Triggerd MDT configuration.

8.1.6.1.2.5.3	Test description

8.1.6.1.2.5.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

UE:

-	None.

Preamble:

-	The UE is in state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

8.1.6.1.2.5.3.2	Test procedure sequence

Table 8.1.6.1.2.5.3.2-1/2 illustrate the downlink power levels and other changing parameters to be applied for the cell at various time instants of the test execution. Row marked "T0" denotes the initial conditions, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.6.1.2.5.3.2-1: Time instances of cell power level in FR1

|  | Parameter | Unit | NR Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -85 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -98 | Power level is such that entry condition for eventL1 is satisfied Ms + Hys < Thresh |

Table 8.1.6.1.2.5.3.2-2: Time instances of cell power level in FR2

|  | Parameter | Unit | NR Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -82 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -100 | Power level is such that entry condition for eventL1 is satisfied Ms + Hys < Thresh |

Table 8.1.6.1.2.5.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits a LoggedMeasurementConfiguration message with reportType is set to eventTriggered and eventType is set to eventL1, to configure the UE to perform logging of measurement results while in RRC_IDLE. | <-- | NR RRC: LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| 3 | Wait 30s to allow UE to activate logging. | - | - | - | - |
| 4-6 | Steps 1 to 3 of the generic procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 7 | Check: Does the UE include the IE logMeasAvailable in the RRCSetupComplete message? | --> | NR RRC: RRCSetupComplete 5GMM: SERVICE REQUEST | 1 | F |
| 8-11 | Steps 5 to 8 of the generic procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 12 | The SS transmits a UEInformationRequest message with logMeasReportReq present. | <-- | NR RRC: UEInformationRequest | - | - |
| 13 | Check: Does the UE transmit a UEInformationResponse message on SRB1 including logMeasReport? | --> | NR RRC: UEInformationResponse | 1 | F |
| 14 | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| 15 | The SS changes NR Cell 1 levels according to the row "T1" in table 8.1.6.1.2.5.3.2-1/2. | - | - | - | - |
| 16 | Wait 30s to allow UE to activate logging. | - | - | - | - |
| 17-19 | Steps 1 to 3 of the generic procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are executed. | - | - | - | - |
| 20 | Check: Does the UE include the IE logMeasAvailable in the RRCSetupComplete message? | --> | NR RRC: RRCSetupComplete 5GMM: SERVICE REQUEST | 1 | P |
| 21-24 | Steps 5 to 8 of of the generic procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 25 | The SS transmits a UEInformationRequest message. | <-- | NR RRC: UEInformationRequest | - | - |
| 26 | Check: Does the UE transmit a UEInformationResponse message on SRB2? | --> | NR RRC: UEInformationResponse | 1 | P |

8.1.6.1.2.5.3.3	Specific message contents

Table 8.1.6.1.2.5.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.1.6.1.2.5.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-5AA with condition EVENTL1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| loggedMeasurementConfiguration-r16 SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventType-r16 CHOICE { |  |  |  |
| eventL1 SEQUENCE { |  |  |  |
| l1-Threshold CHOICE { |  |  |  |
| rsrp | 69 | -87dBm | FR1 |
|  | 65 + Delta(NRfs) | SS RSRP corresponding to -91dBm | FR2 |
| } |  |  |  |
| hysteresis | 0 |  |  |
| timeToTrigger | ms100 | 0.1s |  |
| } |  |  |  |
| loggingInterval-r16 | ms2560 | 2.56s |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Delta(NRfs) is derived based on calibration procedure defined in the TS 38.508-1 [4], clause 6.1.3.3 |  |  |  |

Table 8.1.6.1.2.5.3.3-2: Void

Table 8.1.6.1.2.5.3.3-3: UEInformationRequest (step 12 and step 25, Table 8.1.6.1.2.5.3.2-3)

Derivation path: TS 38.508-1 [4], Table 4.6.1-32A with condition LOG

Table 8.1.6.1.2.5.3.3-4: UEInformationResponse (step 13, Table 8.1.6.1.2.5.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| logMeasReport-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.2.5.3.3-5: RRCSetupComplete (steps 7 and 20, Table 8.1.6.1.2.5.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |
| nonCriticalExtension { |  |  |  |
| ue-MeasurementsAvailable-r16 | UE-MeasurementsAvailable-r16 with condition LOG |  | Step 20 |
| ue-MeasurementsAvailable-r16 | UE-logMeasurements-NotAvailable | Table 8.1.6.1.2.5.3.3-5A | Step 7 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.2.5.3.3-5A: UE-logMeasurements-NotAvailable (step 7, Table 8.1.6.1.2.5.3.2-3)

| Derivation path: TS 38.508-1[4], Table 4.6.5-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE-MeasurementsAvailable-r16 ::= SEQUENCE { |  |  |  |  |
| logMeasAvailable-r16 |  | Not Present |  |  |

Table 8.1.6.1.2.5.3.3-6: UEInformationResponse (step 26, Table 8.1.6.1.2.5.3.2-3)

| Derivation path: TS 38.508-1[4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| logMeasReport-r16 SEQUENCE { |  |  |  |
| absoluteTimeStamp-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceReference-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 SEQUENCE { |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |
| traceId-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |
| traceRecordingSessionRef-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| tce-Id-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r16 SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-r16 SEQUENCE { | At least 2 entries where at least one entry complies to entry with index ‘x’ below. SS records the relativeTimeStamp value for each entry |  |  |
| LogMeasInfo-r16[x] SEQUENCE { |  | entry x |  |
| locationInfo-r16 | Not checked |  |  |
| relativeTimeStamp-r16 | SS record the value |  |  |
| servCellIdentity-r16 | Same as NR Cell 1 |  |  |
| measResultServCell-16 SEQUENCE { |  |  |  |
| resultsSSB-Cell-r16 | MeasQuantityResults of NR Cell 1 |  |  |
| resultsSSB SEQUENCE { |  |  |  |
| best-ssb-Index | Not checked |  |  |
| best-ssb-Results | Not checked |  |  |
| numberOfGoodSSB | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells-r16 | Any allowed value |  |  |
| anyCellSelectionDetected-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| logMeasAvailable-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

###### 8.1.6.1.2.6	Logged MDT/ RRC_IDLE / Logging and reporting / event-based trigger/ out-of-coverage

8.1.6.1.2.6.1	Test Purpose (TP)

(1)

with {UE has received LoggedMeasurementConfiguration message with reportType is set to eventTriggered and eventType is set to outOfCoverage }

ensure that {

when { T330 is running }

then { UE performs the logging at regular time intervals as defined by the loggingInterval in VarLogMeasConfig only when the UE is in any cell selection state }

}

(2)

with {UE has received LoggedMeasurementConfiguration message with reportType is set to eventTriggered and eventType is set to outOfCoverage }

ensure that {

when { T330 is running }

then { UE performs the logging immediately upon transitioning from the any cell selection state to the camped normally state }

}

8.1.6.1.2.6.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clauses 5.3.3.4, 5 5.5a.1.3, 5.5a.3.2, 5.7.10.3 and 6.2.2. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.3.3.4]

The UE shall perform the following actions upon reception of the RRCSetup:

…

5>	set the content of RRCSetupComplete message as follows:

…

2>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable in the RRCSetupComplete message;

2>	if the UE has Bluetooth logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailableBT in the RRCSetupComplete message;

2>	if the UE has WLAN logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailableWLAN in the RRCSetupComplete message;

…

[TS 38.331, clause 5.5a.1.3]

Upon receiving the LoggedMeasurementConfiguration message the UE shall:

1>	discard the logged measurement configuration as well as the logged measurement information as specified in 5.5a.2;

1>	store the received loggingDuration, reportType and areaConfiguration, if included, in VarLogMeasConfig;

1>	if the LoggedMeasurementConfiguration message includes plmn-IdentityList:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN as well as the PLMNs included in plmn-IdentityList;

1>	else:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN;

1>	store the received absoluteTimeInfo, traceReference, traceRecordingSessionRef, and tce-Id in VarLogMeasReport;

1>	store the received bt-NameList, if included, in VarLogMeasConfig;

1>	store the received wlan-Namelist, if included, in VarLogMeasConfig;

1>	store the received sensor-Namelist, if included, in VarLogMeasConfig;

1>	start timer T330 with the timer value set to the loggingDuration;

[TS 38.331, clause5.5a.3.2]

While T330 is running, the UE shall:

1>	perform the logging in accordance with the following:

2>	if the reportType is set to periodical in the VarLogMeasConfig:

3>	if the UE is camping normally on an NR cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and, if the cell is part of the area indicated by areaConfiguration if configured in VarLogMeasConfig:

4>	perform the logging at regular time intervals, as defined by the loggingInterval in the VarLogMeasConfig;

2>	else if the reportType is set to eventTriggered, and eventType is set to outOfCoverage:

3>	perform the logging at regular time intervals as defined by the loggingInterval in VarLogMeasConfig only when the UE is in any cell selection state;

3>	perform the logging immediately upon transitioning from the any cell selection state to the camped normally state;

2>	else if the reportType is set to eventTriggered and eventType is set to eventL1:

3>	if the UE is camping normally on an NR cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and, if the cell is part of the area indicated by areaConfiguration if configured in VarLogMeasConfig;

4>	perform the logging at regular time intervals as defined by the loggingInterval in VarLogMeasConfig only when the conditions indicated by the eventL1 are met;

2>	when performing the logging:

3>	set the relativeTimeStamp to indicate the elapsed time since the moment at which the logged measurement configuration was received;

3>	if detailed location information became available during the last logging interval, set the content of the locationInfo as in 5.3.3.7:

3>	if the UE is in any cell selection state (as specified in TS 38.304 [20]):

4>	set anyCellSelectionDetected to indicate the detection of no suitable or no acceptable cell found;

4>	set the servCellIdentity to indicate global cell identity of the last logged cell that the UE was camping on;

4>	set the measResultServingCell to include the quantities of the last logged cell the UE was camping on;

3>	else:

4>	set the servCellIdentity to indicate global cell identity of the cell the UE is camping on;

4>	set the measResultServingCell to include the quantities of the cell the UE is camping on;

4>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements that became available during the last logging interval for at most the following number of neighbouring cells: 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies per RAT and according to the following:

5>	for each neighbour cell included, include the optional fields that are available;

NOTE:	The UE includes the latest results of the available measurements as used for cell reselection evaluation in RRC_IDLE or RRC_INACTIVE, which are performed in accordance with the performance requirements as specified in TS 38.133 [14].

2>	when the memory reserved for the logged measurement information becomes full, stop timer T330 and perform the same actions as performed upon expiry of T330, as specified in 5.5a.1.4.

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if the logMeasReportReq is present and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3>	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3>	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3>	include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3>	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3>	include the logMeasInfoList and set it to include one or more entries from VarLogMeasReport starting from the entries logged first;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

3>	if the VarLogMeasReport includes one or more additional logged Bluetooth measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailableBT;

3>	if the VarLogMeasReport includes one or more additional logged WLAN measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailableWLAN;

…

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1.

[TS 38.331, clause 6.2.2]

–	LoggedMeasurementConfiguration

The LoggedMeasurementConfiguration message is used to perform logging of measurement results while in RRC_IDLE or RRC_INACTIVE. It is used to transfer the logged measurement configuration for network performance optimisation.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

LoggedMeasurementConfiguration message

-- ASN1START

-- TAG-LOGGEDMEASUREMENTCONFIGURATION-START

LoggedMeasurementConfiguration-r16 ::=  SEQUENCE {

criticalExtensions                      CHOICE {

loggedMeasurementConfiguration-r16      LoggedMeasurementConfiguration-r16-IEs,

criticalExtensionsFuture                SEQUENCE {}

}

}

LoggedMeasurementConfiguration-r16-IEs ::=  SEQUENCE {

traceReference-r16                          TraceReference-r16,

traceRecordingSessionRef-r16                OCTET STRING (SIZE (2)),

tce-Id-r16                                  OCTET STRING (SIZE (1)),

absoluteTimeInfo-r16                        AbsoluteTimeInfo-r16,

areaConfiguration-r16                       AreaConfiguration-r16                    OPTIONAL,  --Need R

plmn-IdentityList-r16                       PLMN-IdentityList2-r16                   OPTIONAL,  --Need R

bt-NameList-r16                             SetupRelease {BT-NameList-r16}           OPTIONAL,  --Need M

wlan-NameList-r16                           SetupRelease {WLAN-NameList-r16}         OPTIONAL,  --Need M

sensor-NameList-r16                         SetupRelease {Sensor-NameList-r16}       OPTIONAL,  --Need M

loggingDuration-r16                         LoggingDuration-r16,

reportType                                  CHOICE {

periodical                                  LoggedPeriodicalReportConfig-r16,

eventTriggered                              LoggedEventTriggerConfig-r16,

...

},

lateNonCriticalExtension                    OCTET STRING                             OPTIONAL,

nonCriticalExtension                        SEQUENCE {}                              OPTIONAL

}

LoggedPeriodicalReportConfig-r16 ::=            SEQUENCE {

loggingInterval-r16                             LoggingInterval-r16,

...

}

LoggedEventTriggerConfig-r16 ::=                SEQUENCE {

eventType-r16                                   EventType-r16,

loggingInterval-r16                             LoggingInterval-r16,

...

}

EventType-r16 ::= CHOICE {

outOfCoverage     NULL,

eventL1           SEQUENCE {

l1-Threshold      MeasTriggerQuantity,

hysteresis        Hysteresis,

timeToTrigger     TimeToTrigger

},

...

}

-- TAG-LOGGEDMEASUREMENTCONFIGURATION-STOP

-- ASN1STOP

LoggedMeasurementConfiguration field descriptions

absoluteTimeInfo Indicates the absolute time in the current cell.

areaConfiguration Used to restrict the area in which the UE performs measurement logging to cells broadcasting either one of the included cell identities or one of the included tracking area codes/ frequencies.

eventType The value outOfCoverage indicates the UE to perform logging of measurements when the UE enters any cell selection state, and the value eventL1 indicates the UE to perform logging of measurements when the triggering condition (similar as event A2 as specified in 5.5.4.3) as configured in the event is met for the camping cell in camped normally state.

plmn-IdentityList Indicates a set of PLMNs defining when the UE performs measurement logging as well as the associated status indication and information retrieval i.e. the UE performs these actions when the RPLMN is part of this set of PLMNs.

tce-Id Parameter Trace Collection Entity Id: See TS 32.422 [52].

traceRecordingSessionRef Parameter Trace Recording Session Reference: See TS 32.422 [52].

reportType Parameter configures the type of MDT configuration, specifically Periodic MDT conifguraiton or Event Triggerd MDT configuration.

8.1.6.1.2.6.3	Test description

8.1.6.1.2.6.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

UE:

-	None.

Preamble:

-	The UE is in state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

8.1.6.1.2.6.3.2	Test procedure sequence

Table 8.1.6.1.2.6.3.2-1/2 illustrate the downlink power levels and other changing parameters to be applied for the cell at various time instants of the test execution. Row marked "T0" denotes the initial conditions, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.6.1.2.6.3.2-1: Time instances of cell power level in FR1

|  | Parameter | Unit | NR Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -85 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | Off | Make UE go to any cell selection state |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -85 | Cell 1 become suitable cell |

Table 8.1.6.1.2.6.3.2-2: Time instances of cell power level in FR2

|  | Parameter | Unit | NR Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -82 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | Off | Make UE go to any cell selection state |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -82 | Cell 1 become suitable cell |

Table 8.1.6.1.2.6.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits a LoggedMeasurementConfiguration message with reportType is set to eventTriggered and eventType is set to outOfCoverage, to configure the UE to perform logging of measurement results. | <-- | NR RRC: LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| 3 | Wait 30s to allow UE to activate logging. | - | - | - | - |
| 4-6 | Steps 1 to 3 of the generic procedure in TS 38.508-1 [4], Table 4.5.4.2-3 are executed. | - | - | - | - |
| 7 | Check: Does the UE include the IE logMeasAvailable in the RRCSetupComplete message? | --> | NR RRC: RRCSetupComplete 5GMM: SERVICE REQUEST | 1 | F |
| 8-11 | Steps 5 to 8 of the generic procedure in TS 38.508-1 [4], Table 4.5.4.2-3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 12 | The SS transmits a UEInformationRequest message with logMeasReportReq present. | <-- | NR RRC: UEInformationRequest | - | - |
| 13 | Check: Does the UE transmit a UEInformationResponse message on SRB1 including logMeasReport? | --> | NR RRC: UEInformationResponse | 1 | F |
| 14 | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| 15 | The SS changes NR Cell 1 level according to the row "T1" in table 8.1.6.1.2.6.3.2-1/2. | - | - | - | - |
| 16 | Wait 30s to allow UE to activate logging. | - | - | - | - |
| 17 | The SS changes NR Cell 1 level according to the row "T2" in table 8.1.6.1.2.6.3.2-1/2. | - | - | - | - |
| 18 | Wait 10s to allow UE to go to camp normaly state and activate logging. | - | - | - | - |
| 19 | Steps 1 to 3 of the generic procedure in TS 38.508-1 [4], Table 4.5.4.2-3 are executed. | - | - | - | - |
| 20 | Check: Does the UE include the IE logMeasAvailable in the RRCSetupComplete message? | --> | NR RRC: RRCSetupComplete 5GMM: SERVICE REQUEST | 1, 2 | P |
| 21-24 | Steps 5 to 8 of the generic procedure in TS 38.508-1 [4], Table 4.5.4.2-3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 25 | The SS transmits a UEInformationRequest message to get logMeasReport. | <-- | NR RRC: UEInformationRequest | - | - |
| 26 | Check: Does the UE transmit a UEInformationResponse message on SRB2 with logMeasReport including at least one LogMeasInfo for any cell selection state and one LogMeasInfo for transitioning from the any cell selection state to the camped normally state? | --> | NR RRC: UEInformationResponse | 1, 2 | P |

8.1.6.1.2.6.3.3	Specific message contents

Table 8.1.6.1.2.6.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.1.6.1.2.6.3.2-3)

Derivation path: TS 38.508-1 [4], Table 4.6.1-5AA with condition EVENT and OUT-OF-COVERAGE

Table 8.1.6.1.2.6.3.3-2: Void

Table 8.1.6.1.2.6.3.3-3: RRCSetupComplete (step 7, Table 8.1.6.1.2.6.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-22. |
| --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { | Checked | Note 1 |  |
| ue-MeasurementsAvailable-r16 SEQUENCE { |  |  | If nonCriticalExtension is present |
| logMeasAvailable-r16 | Not present |  | If ue-MeasurementsAvailable-r16 is present |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| If nonCriticalExtension and ue-MeasurementsAvailable-r16 are present | Contents of ue-MeasurementsAvailable-r16 shall be checked only if nonCriticalExtension and ue-MeasurementsAvailable-r16 are present |
| Note 1: If the UE has no underlying information to report to the network, this information element may be absent. If this information element and ue-MeasurementsAvailable-r16 are present, further check on the contents of the ue-MeasurementsAvailable-r16 shall be performed. |  |

Table 8.1.6.1.2.6.3.3-4: UEInformationRequest (steps 12 and 25, Table 8.1.6.1.2.6.3.2-3)

Derivation path: TS 38.508-1 [4], Table 4.6.1-32A with condition LOG

Table 8.1.6.1.2.6.3.3-5: UEInformationResponse (step 13, Table 8.1.6.1.2.6.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| logMeasReport-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.2.6.3.3-6: RRCSetupComplete (steps 20, Table 8.1.6.1.2.6.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |
| nonCriticalExtension { |  |  |  |
| ue-MeasurementsAvailable-r16 | UE-MeasurementsAvailable-r16 with condition LOG |  | Step 20 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.2.6.3.3-7: UEInformationResponse (step 26, Table 8.1.6.1.2.6.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| logMeasReport-r16 SEQUENCE { |  |  |  |
| absoluteTimeStamp-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceReference-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 SEQUENCE { |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |
| traceId-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |
| traceRecordingSessionRef-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| tce-Id-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r16 SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-r16 SEQUENCE { | At least 2 entries where at least one entry complies to entry with index ‘x’ below. SS records the relativeTimeStamp value for each entry. And the relative TimeStamp of first entry is more than 10s before the last one. |  |  |
| LogMeasInfo-r16[x] SEQUENCE { |  | entry x |  |
| locationInfo-r16 | Not checked |  |  |
| relativeTimeStamp-r16 | SS record the value |  |  |
| servCellIdentity-r16 | Same as NR Cell 1 |  |  |
| measResultServCell-16 SEQUENCE { |  |  |  |
| resultsSSB-Cell-r16 | MeasQuantityResults of NR Cell 1 |  |  |
| resultsSSB SEQUENCE { |  |  |  |
| best-ssb-Index | Not checked |  |  |
| best-ssb-Results | Not checked |  |  |
| numberOfGoodSSB | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells-r16 | Any allowed value |  |  |
| anyCellSelectionDetected-r16 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| logMeasAvailable-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

###### 8.1.6.1.2.7	Logged MDT / Logging and reporting / Indication of logged measurements at NR reestablishment

8.1.6.1.2.7.1	Test Purpose (TP)

(1)

with { UE has one or more logged measurement entries stored in VarLogMeasReport }

ensure that {

when { receiving RRCReestablishment message }

then { UE includes the logMeasAvailable IE in the RRCReestablishmentComplete message }

}

8.1.6.1.2.7.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clauses 5.3.7.5. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.3.7.5]

The UE shall:

1>	stop timer T301;

1>	consider the current cell to be the PCell;

1>	store the nextHopChainingCount value indicated in the RRCReestablishment message;

1>	update the KgNB key based on the current KgNB key or the NH, using the stored nextHopChainingCount value, as specified in TS 33.501 [11];

1>	derive the KRRCenc and KUPenc keys associated with the previously configured cipheringAlgorithm, as specified in TS 33.501 [11];

1>	derive the KRRCint and KUPint keys associated with the previously configured integrityProtAlgorithm, as specified in TS 33.501 [11].

1>	request lower layers to verify the integrity protection of the RRCReestablishment message, using the previously configured algorithm and the KRRCint key;

1>	if the integrity protection check of the RRCReestablishment message fails:

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure', upon which the procedure ends;

1>	configure lower layers to resume integrity protection for SRB1 using the previously configured algorithm and the KRRCint key immediately, i.e., integrity protection shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	configure lower layers to resume ciphering for SRB1 using the previously configured algorithm and, the KRRCenc key immediately, i.e., ciphering shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	release the measurement gap configuration indicated by the measGapConfig, if configured;

1>	set the content of RRCReestablishmentComplete message as follows:

2>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable in the RRCReestablishmentComplete message;

3>	if Bluetooth measurement results are included in the logged measurements the UE has available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

4>	include the logMeasAvailableBT in the RRCReestablishmentComplete message;

3>	if WLAN measurement results are included in the logged measurements the UE has available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

4>	include the logMeasAvailableWLAN in the RRCReestablishmentComplete message;

2>	if the UE has connection establishment failure or connection resume failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

3>	include connEstFailInfoAvailable in the RRCReestablishmentComplete message;

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report; or

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report of TS 36.331 [10] and if the UE is capable of cross-RAT RLF reporting and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 36.331 [10]:

3>	include rlf-InfoAvailable in the RRCReestablishmentComplete message;

1>	submit the RRCReestablishmentComplete message to lower layers for transmission;

1>	the procedure ends.

8.1.6.1.2.7.3	Test description

8.1.6.1.2.7.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 and NR Cell 2.

-	System information combination NR-2 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

-	None.

Preamble:

-	The UE is in state 3N-A on NR Cell 1 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

8.1.6.1.2.7.3.2	Test procedure sequence

Table 8.1.6.1.2.7.3.2-1/2 illustrate the downlink power levels and other changing parameters to be applied for the cell at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.6.1.2.7.3.2-1: Time instances of cell power level for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 |
| --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -85 | -91 |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | Off | -85 |

Table 8.1.6.1.2.7.3.2-2: Time instances of cell power level for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 |
| --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -91 |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | Off | -82 |

Table 8.1.6.1.2.7.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits a LoggedMeasurementConfiguration message with reportType is set to ‘periodical’ and LoggingInterval is set to ‘ms10240’, to configure the UE to perform logging of measurement results while in RRC_IDLE. | <-- | LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| 3 | Wait 30s to allow UE to activate logging. | - | - | - | - |
| 4-11 | Steps 1 to 8 of the generic procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure on NR Cell 1. | - | - | - | - |
| 12 | The SS changes NR Cell 1 and NR Cell 2 levels according to the row "T1" in table 8.1.6.1.2.7.3.2-1 for FR1 or table 8.1.6.1.2.7.3.2-2 for FR2. | - | - | - | - |
| 13 | The UE sends RRCReestablishmentRequest message on NR Cell 2. | --> | NR RRC: RRCReestablishmentRequest | - | - |
| 14 | The SS transmits RRCReestablishment message. | <-- | NR RRC: RRCReestablishment | - | - |
| 15 | Check: Does the UE transmit RRCReestablishmentComplete message including logMeasAvailable IE? | --> | NR RRC: RRCReestablishmentComplete | 1 | P |
| 16 | The SS transmits an RRCReconfigurationmessage to resume existing radio bearer. | <-- | NR RRC: RRCReconfiguration | - | - |
| 17 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationtComplete | - | - |
| 18 | The SS sends an UEInformationRequest message to get logMeasReport and rlf-Report. | <-- | NR RRC: UEInformationRequest | - | - |
| 19 | The UE sends an UEInformationResponse message on SRB2 including logMeasReport and rlf-Report. | --> | NR RRC: UEInformationResponse | - | - |

8.1.6.1.2.7.3.3	Specific message contents

Table 8.1.6.1.2.7.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.1.6.1.2.7.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-5AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| loggedMeasurementConfiguration-r16 SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| loggingInterval-r16 | ms10240 | 10.24 seconds |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.2.7.3.3-2: RRCReestablishmentComplete (step 15, Table 8.1.6.1.2.7.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReestablishmentComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ue-MeasurementsAvailable-r16 | UE-MeasurementsAvailable-r16 with condition LOG and RLF |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.2.7.3.3-3: UEInformationRequest (step 18, Table 8.1.6.1.2.7.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-32A with condition LOG and RLF

###### 8.1.6.1.2.8	Logged MDT / Logging and reporting / Reporting at RRC reconfiguration

8.1.6.1.2.8.1	Test Purpose (TP)

(1)

with { UE received LoggedMeasurementConfiguration message and has stored logMeasReport in VarLogMeasReport }

ensure that {
  when { UE receives an RRCReconfiguration message including a reconfigurationWithSyn }

then { UE transmits an RRCReconfigurationComplete message with logMeasAvailable set to true }

}

(2)

with { UE having indicated availability of logged measurements in RRCReconfigurationComplete message }

ensure that {

when { UE has completed intra NR handover procedure and receives a UEInformationRequest message with logMeasReportReq present }

then { UE transmits an UEInformationResponse message including logMeasReport }

}

8.1.6.1.2.8.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clauses 5.3.5.3 and and 5.7.10.3. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.3.5.3]

The UE shall perform the following actions upon reception of the RRCReconfiguration, or upon execution of the conditional reconfiguration (CHO or CPC):

1>	if the RRCReconfiguration is applied due to a conditional reconfiguration execution upon cell selection while timer T311 is running, as defined in 5.3.7.3:

2>	remove all the entries within VarConditionalReconfig, if any;

…

1>	set the content of the RRCReconfigurationComplete message as follows:

2>	if the RRCReconfiguration includes the masterCellGroup containing the reportUplinkTxDirectCurrent:

3>	include the uplinkTxDirectCurrentList for each MCG serving cell with UL;

3>	include uplinkDirectCurrentBWP-SUL for each MCG serving cell configured with SUL carrier, if any, within the uplinkTxDirectCurrentList;

2>	if the RRCReconfiguration includes the secondaryCellGroup containing the reportUplinkTxDirectCurrent:

3>	include the uplinkTxDirectCurrentList for each SCG serving cell with UL;

3>	include uplinkDirectCurrentBWP-SUL for each SCG serving cell configured with SUL carrier, if any, within the uplinkTxDirectCurrentList;

2>	if the RRCReconfiguration message includes the mrdc-SecondaryCellGroupConfig with mrdc-SecondaryCellGroup set to eutra-SCG:

3>	include in the eutra-SCG-Response the E-UTRA RRCConnectionReconfigurationComplete message in accordance with TS 36.331 [10] clause 5.3.5.3;

2> if the RRCReconfiguration message includes the mrdc-SecondaryCellGroupConfig with mrdc-SecondaryCellGroup set to nr-SCG:

3>	include in the nr-SCG-Response the RRCReconfigurationComplete message;

2>	if the RRCReconfiguration message was included in an RRCResume message:

3>	include the RRCReconfigurationComplete message in the nr-SCG-Response within the scg-Response in the RRCResumeComplete message;

2>	if the RRCReconfiguration message was included in E-UTRA RRCConnectionResume message:

3>	include the RRCReconfigurationComplete message in the E-UTRA MCG RRC message RRCConnectionResumeComplete in accordance with TS 36.331 [10], clause 5.3.3.4a;

2>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable in the RRCReconfigurationComplete message;

3>	if Bluetooth measurement results are included in the logged measurements the UE has available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

4>	include the logMeasAvailableBT in the RRCReconfigurationComplete message;

3>	if WLAN measurement results are included in the logged measurements the UE has available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

4>	include the logMeasAvailableWLAN in the RRCReconfigurationComplete message;

…

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if the logMeasReportReq is present and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3>	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3>	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3>	include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3>	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3>	include the logMeasInfoList and set it to include one or more entries from VarLogMeasReport starting from the entries logged first;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

3>	if the VarLogMeasReport includes one or more additional logged Bluetooth measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailableBT;

3>	if the VarLogMeasReport includes one or more additional logged WLAN measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailableWLAN;

…

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1.

8.1.6.1.2.8.3	Test description

8.1.6.1.2.8.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 and NR Cell 2

-	System information combination NR-2 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

-	None.

Preamble:

-	The UE is in state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3 on NR Cell 1.

8.1.6.1.2.8.3.2	Test procedure sequence

Table 8.1.6.1.2.8.3.2-1/2 illustrate the downlink power levels and other changing parameters to be applied for the cell at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while the configuration marked "T1" shall be applied as described in the texts in this clause.

Table 8.1.6.1.2.8.3.2-1: Power levels in FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -85 | -91 | Power levels are such that entry condition for event A3 is not satisfied for NR Cell 2 |
| T1 | SS/PBCH SSS EPRE | dBm/ SCS | -85 | -79 | Power levels are such that entry condition for event A3 is satisfied for NR Cell 2 |

Table 8.1.6.1.2.8.3.2-2: Power levels in FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -91 | -100 | Power levels are such that entry condition for event A3 is not satisfied for NR Cell 2 |
| T1 | SS/PBCH SSS EPRE | dBm/ SCS | -91 | -82 | Power levels are such that entry condition for event A3 is satisfied for NR Cell 2 |

Table 8.1.6.1.2.8.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a LoggedMeasurementConfiguration message on NR Cell 1. | <-- | NR RRC: LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits a RRCRelease message to release the RRC connection. | <-- | NR RRC: RRCRelease | - | - |
| 3 | Wait 30 seconds for the UE to perform the logging at regular time intervals. | - | - | - | - |
| 4-11 | Steps 1 to 8 of the generic procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 12 | The SS transmits an RRCReconfiguration message including MeasConfig to setup intra NR measurement and reporting for intra-frequency event A3. | <-- | NR RRC: RRCReconfiguration | - | - |
| 13 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 14 | The SS changes NR Cell 1 and NR Cell 2 parameters according to the row "T1" in tables 8.1.6.1.2.8.3.2-1/2. | - | - | - | - |
| 15 | The UE transmits a MeasurementReport message to report event A3 with the measured RSRP value for NR Cell 2. | --> | NR RRC: MeasurementReport | - | - |
| 16 | The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform intra frequency handover to NR Cell 2. | <-- | NR RRC: RRCReconfiguration | - | - |
| 17 | Check: Does the UE transmit an RRCReconfigurationComplete message on NR Cell 2 with logMeasAvailable set to true to confirm the successful completion of the intra frequency handover? | --> | NR RRC: RRCReconfigurationComplete | 1 | P |
| 18 | The SS sends an UEInformationRequest message to get logMeasReport. | <-- | NR RRC: UEInformationRequest | - | - |
| 19 | Check: Does the UE send an UEInformationReponse message including logMeasReport? | --> | NR RRC: UEInformationResponse | 2 | P |

8.1.6.1.2.8.3.3	Specific message contents

Table 8.1.6.1.2.8.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.1.6.1.2.8.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-5AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| loggedMeasurementConfiguration-r16 SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| loggingInterval-r16 | ms10240 | 10.24 seconds |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.2.8.3.3-2: RRCReconfiguration (step 12, Table 8.1.6.1.2.8.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-13, condition NR_MEAS

Table 8.1.6.1.2.8.3.3-3: MeasConfig (Table 8.1.6.1.2.8.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR SEQUENCE { |  |  |  |
| ssbFrequency | ssbFrequency IE equals the ARFCN for NR Cell 1 |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-EventA3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList::= SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.2.8.3.3-4: ReportConfigNR-EventA3 (Table 8.1.6.1.2.8.3.3-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-142 with condition EVENT_A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | 2 | 1 dB (2*0.5 dB) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportAmount | r1 |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | true |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.2.8.3.3-4A: RRCReconfiguration-HO (step 16, Table 8.1.6.1.2.8.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.8.1-1A with condition RBConfig_KeyChange

Table 8.1.6.1.2.8.3.3-5: RRCReconfigurationComplete (step 17, Table 8.1.6.1.2.8.3.2-3)

| Derivation path: TS 38.508-1 [4] Table 4.6.1-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfigurationComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfigurationComplete SEQUENCE { |  |  |  |
| nonCriticalExtension { |  |  |  |
| nonCriticalExtension { |  |  |  |
| nonCriticalExtension { |  |  |  |
| ue-MeasurementsAvailable-r16 | UE-MeasurementsAvailable-r16 with condition LOG |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.2.8.3.3-6: UEInformationRequest (step 18, Table 8.1.6.1.2.8.3.2-3)

Derivation path: TS 38.508-1 [4] Table 4.6.1-32A with condition LOG

Table 8.1.6.1.2.8.3.3-7: UEInformationResponse (step 19, Table 8.1.6.1.2.8.3.2-3)

| Derivation path: TS 38.508-1 [4] Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| logMeasReport-r16 SEQUENCE { |  |  |  |
| absoluteTimeStamp-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceReference-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 SEQUENCE { |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |
| traceId-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |
| traceRecordingSessionRef-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| tce-Id-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r16 SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-r16 SEQUENCE { | At least 2 entries where at least one entry complies to entry with index ‘x’ below. SS records the relativeTimeStamp value for each entry |  |  |
| LogMeasInfo-r16[x] SEQUENCE { |  | entry x |  |
| locationInfo-r16 | Not checked |  |  |
| relativeTimeStamp-r16 | SS record the value |  |  |
| servCellIdentity-r16 | Same as NR Cell 1 |  |  |
| measResultServCell-16 SEQUENCE { |  |  |  |
| resultsSSB-Cell-r16 | MeasQuantityResults of NR Cell 1 |  |  |
| resultsSSB SEQUENCE { |  |  |  |
| best-ssb-Index | Not checked |  |  |
| best-ssb-Results | Not checked |  |  |
| numberOfGoodSSB | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells-r16 SEQUENCE { |  |  |  |
| measResultNeighCellListNR SEQUENCE (SIZE (1..maxFreq)) OF MeasResultLogging2NR-r16 SEQUENCE { | 1 entry |  |  |
| MeasResultLogging2NR-r16[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r16 | Same as NR Cell 2 |  |  |
| measResultListLoggingNR -r16 SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultLoggingNR-r16 SEQUENCE { | 1 entry |  |  |
| MeasResultLoggingNR-r16[1] SEQUENCE { |  | entry 1 |  |
| physCellId-r16 | Same as NR Cell 2 |  |  |
| resultsSSB-Cell-r16 | MeasQuantityResults of NR Cell 2 |  |  |
| numberOfGoodSSB-r16 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCellListEUTRA | Not present |  |  |
| } |  |  |  |
| anyCellSelectionDetected-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| logMeasAvailable-r16 | Not present |  |  |
| logMeasAvailableBT-r16 | Not present |  |  |
| logMeasAvailableWLAN-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.2.8.3.3-8: RRCSetupComplete (step 7, Table 8.1.6.1.2.8.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ue-MeasurementsAvailable-r16 | UE-MeasurementsAvailable-r16 with condition LOG |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

###### 8.1.6.1.2.9	Logged MDT / Location information

8.1.6.1.2.9.1	Test Purpose (TP)

(1)

with { UE received LoggedMeasurementConfiguration message and has stored logMeasReport with detailed

location information in VarLogMeasReport }

ensure that {

when { receiving UEInformationRequest message }

then { UE transmits UEInformationResponse messages with a logMeasReport with locationCoordinate in the locationInfo in the logged measurement entries }

}

8.1.6.1.2.9.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clauses 5.3.3.4, 5 5.5a.1.3, 5.5a.3.2, 5.3.3.7 and 5.7.10.3. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.3.3.4]

The UE shall perform the following actions upon reception of the RRCSetup:

…

1>	set the content of RRCSetupComplete message as follows:

…

2>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable in the RRCSetupComplete message;

2>	if the UE has Bluetooth logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailableBT in the RRCSetupComplete message;

2>	if the UE has WLAN logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailableWLAN in the RRCSetupComplete message;

…

[TS 38.331, clause 5.5a.1.3]

Upon receiving the LoggedMeasurementConfiguration message the UE shall:

1>	discard the logged measurement configuration as well as the logged measurement information as specified in 5.5a.2;

1>	store the received loggingDuration, reportType and areaConfiguration, if included, in VarLogMeasConfig;

1>	if the LoggedMeasurementConfiguration message includes plmn-IdentityList:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN as well as the PLMNs included in plmn-IdentityList;

1>	else:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN;

1>	store the received absoluteTimeInfo, traceReference, traceRecordingSessionRef, and tce-Id in VarLogMeasReport;

1>	store the received bt-NameList, if included, in VarLogMeasConfig;

1>	store the received wlan-Namelist, if included, in VarLogMeasConfig;

1>	store the received sensor-Namelist, if included, in VarLogMeasConfig;

1>	start timer T330 with the timer value set to the loggingDuration;

[TS 38.331, clause 5.5a.3.2]

While T330 is running, the UE shall:

1>	perform the logging in accordance with the following:

…

2>	when performing the logging:

3>	set the relativeTimeStamp to indicate the elapsed time since the moment at which the logged measurement configuration was received;

3>	if location information became available during the last logging interval, set the content of the locationInfo as in 5.3.3.7:

[TS 38.331, clause 5.3.3.7]

3>	if available, set the locationInfo as follows:

4>	if available, set the commonLocationInfo to include the detailed location information;

4>	if available, set the bt-LocationInfo to include the Bluetooth measurement results, in order of decreasing RSSI for Bluetooth beacons;

4>	if available, set the wlan-LocationInfo to include the WLAN measurement results, in order of decreasing RSSI for WLAN APs;

4>	if available, set the sensor-LocationInfo to include the sensor measurement results as follows;

5>	if available, include the sensor-MeasurementInformation;

5>	if available, include the sensor-MotionInformation;

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if the logMeasReportReq is present and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3>	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3>	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3>	include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3>	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3>	include the logMeasInfoList and set it to include one or more entries from VarLogMeasReport starting from the entries logged first;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

3>	if the VarLogMeasReport includes one or more additional logged Bluetooth measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailableBT;

3>	if the VarLogMeasReport includes one or more additional logged WLAN measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailableWLAN;

…

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1.

8.1.6.1.2.9.3	Test description

8.1.6.1.2.9.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 and NR Cell 11.

-	System information combination NR-2 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

-	None.

Preamble:

-	The UE is in state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3 on Cell 1.

-	The UE's positioning engine (e.g. standalone GNSS receiver) should be provided with any necessary stimulus to allow it to provide the position. This shall be done by use of the test function Update UE Location Information defined in TS 38.509 [6] clause5.6.2. Or in addition any other suitable method may also be used.

8.1.6.1.2.9.3.2	Test procedure sequence

Table 8.1.6.1.2.9.3.2-1/2 illustrate the downlink power levels and other changing parameters to be applied for the cell at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.6.1.2.9.3.2-1: Time instances of cell power level for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 11 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -78 | -85 | NR Cell 1 becomes the highest ranked cell. |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -85 | -78 | NR Cell 11 becomes the highest ranked cell. |

Table 8.1.6.1.2.9.3.2-2: Time instances of cell power level for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 11 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -91 | NR Cell 1 becomes the highest ranked cell. |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -91 | -82 | NR Cell 11 becomes the highest ranked cell. |

Table 8.1.6.1.2.9.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits a LoggedMeasurementConfiguration message including to configure the UE to perform logging of measurement results while in RRC_IDLE. | <-- | NR RRC: LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| 3 | Wait 5s to allow UE to activate logging. | - | - | - | - |
| 4 | The SS changes Cell 1 and Cell 11 levels according to the row "T1" in table 8.1.6.1.2.9.3.2-1/2. | - | - | - | - |
| 5 | The UE transmits an RRCSetupRequest message on Cell 11 to initiate a tracking area update procedure. | --> | NR RRC: RRCSetupRequest | - | - |
| 6 | SS transmit an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 7 | The UE includes the IE logMeasAvailable in the RRCSetupComplete message. | --> | NR RRC: RRCSetupComplete 5GMM: REGISTRATION REQUEST | - | - |
| 8-10 | Steps 4 to 6a1 of generic test procedure in TS 38.508-1 [4] Table 4.9.5.2.2-1 are performed on Cell 11. NOTE: The UE performs a REGISTRATION procedure update the registration of the actual tracking area and the RRC connection is released. | - | - | - | - |
| 11-18 | Steps 1 to 8 of the generic procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 19 | The SS transmits a UEInformationRequest message on Cell 11. | <-- | NR RRC: UEInformationRequest | - | - |
| 20 | Check: Does the UE transmit a UEInformationResponse message where the IE CommonLocationInfo is present? | --> | NR RRC: UEInformationResponse | 1 | P |

8.1.6.1.2.9.3.3	Specific message contents

Table 8.1.6.1.2.9.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.1.6.1.2.9.3.2-3)

Derivation path: TS 38.508-1 [4], Table 4.6.1-5AA with condition PERIODICAL

Table 8.1.6.1.2.9.3.3-2: RRCSetupComplete (step 7, Table 8.1.6.1.2.9.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ue-MeasurementsAvailable-r16 | UE-MeasurementsAvailable-r16 with condition LOG |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.2.9.3.3-3: UEInformationRequest (step 19, Table 8.1.6.1.2.9.3.2-3)

Derivation path: TS 38.508-1 [4], Table 4.6.1-32A with condition LOG

Table 8.1.6.1.2.9.3.3-4: UEInformationResponse (step 20, Table 8.1.6.1.2.9.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| logMeasReport-r16 SEQUENCE { |  |  |  |
| absoluteTimeStamp-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceReference-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 SEQUENCE { |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |
| traceId-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |
| traceRecordingSessionRef-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| tce-Id-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r16 SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-r16 SEQUENCE { | At least 1 entrie where at least one entry complies to entry with index ‘x’ below. |  |  |
| LogMeasInfo-r16[x] SEQUENCE { |  | entry x |  |
| locationInfo-r16 SEQUENCE { |  |  |  |
| commonLocationInfo-r16 SEQUENCE { |  |  |  |
| gnss-TOD-msec-r16 | Not checked |  |  |
| locationTimestamp-r16 | Not checked |  |  |
| locationCoordinate-r16 | OCTET STRING including LocationCoordinates message according to TS 37.355 |  |  |
| locationError-r16 | Not checked |  |  |
| locationSource-r16 | Not checked |  |  |
| velocityEstimate-r16 | Not checked |  |  |
| } |  |  |  |
| bt-LocationInfo-r16 | Not present |  |  |
| wlan-LocationInfo-r16 | Not present |  |  |
| sensor-LocationInfo-r16 | Not present |  |  |
| } |  |  |  |
| relativeTimeStamp-r16 | SS record the value |  |  |
| servCellIdentity-r16 | Same as Cell 1 |  |  |
| measResultServCell-16 SEQUENCE { |  |  |  |
| resultsSSB-Cell-r16 | MeasQuantityResults of Cell 1 |  |  |
| resultsSSB SEQUENCE { |  |  |  |
| best-ssb-Index | Not checked |  |  |
| best-ssb-Results | Not checked |  |  |
| numberOfGoodSSB | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells-r16 SEQUENCE { |  |  |  |
| measResultNeighCellListNR SEQUENCE (SIZE (1..maxFreq)) OF MeasResultLogging2NR-r16 SEQUENCE { | 1 entry |  |  |
| MeasResultLogging2NR-r16[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r16 | Same as Cell 11 |  |  |
| measResultListLoggingNR -r16 SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultLoggingNR-r16 SEQUENCE { | 1 entry |  |  |
| MeasResultLoggingNR-r16[1] SEQUENCE { |  | entry 1 |  |
| physCellId-r16 | Same as Cell 11 |  |  |
| resultsSSB-Cell-r16 | MeasQuantityResults of Cell 11 |  |  |
| numberOfGoodSSB-r16 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCellListEUTRA | Not present |  |  |
| } |  |  |  |
| anyCellSelectionDetected-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| logMeasAvailable-r16 | Not present |  |  |
| logMeasAvailableBT-r16 | Not present |  |  |
| logMeasAvailableWLAN-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

###### 8.1.6.1.2.10	Logged MDT / Maintaining logged measurement configuration / UE mobility

8.1.6.1.2.10.1	Test Purpose (TP)

(1)

with { UE received LoggedMeasurementConfiguration message and has stored logMeasReport in VarLogMeasReport }

ensure that {

when { UE reselected to a cell belong to non MDT PLMN }

then { UE does not indicate availability of Logged MDT measurements }

}

(2)

with { UE received LoggedMeasurementConfiguration message and has stored logMeasReport in VarLogMeasReport }

ensure that {

when { UE reselects to a cell belong to non MDT PLMN }

then { UE suspends Logged MDT measurements }

}

(3)

with { UE received LoggedMeasurementConfiguration message and has reselected to a cell belong to non MDT PLMN }

ensure that {

when { UE returns to a cell belong to MDT PLMN }

then { UE indicate availability of Logged MDT measurements }

}

(4)

with { UE received LoggedMeasurementConfiguration message and has reselected to a cell belong to non MDT PLMN }

ensure that {

when { UE returns to a cell belong to MDT PLMN }

then { UE resumes Logged MDT measurements }

}

8.1.6.1.2.10.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clauses 5.3.3.4, 5 5.5a.1.3 and 5.7.10.3. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.3.3.4]

The UE shall perform the following actions upon reception of the RRCSetup:

…

1>	set the content of RRCSetupComplete message as follows:

…

2>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable in the RRCSetupComplete message;

2>	if the UE has Bluetooth logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailableBT in the RRCSetupComplete message;

2>	if the UE has WLAN logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailableWLAN in the RRCSetupComplete message;

…

[TS 38.331, clause 5.5a.1.3]

Upon receiving the LoggedMeasurementConfiguration message the UE shall:

1>	discard the logged measurement configuration as well as the logged measurement information as specified in 5.5a.2;

1>	store the received loggingDuration, reportType and areaConfiguration, if included, in VarLogMeasConfig;

1>	if the LoggedMeasurementConfiguration message includes plmn-IdentityList:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN as well as the PLMNs included in plmn-IdentityList;

1>	else:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN;

1>	store the received absoluteTimeInfo, traceReference, traceRecordingSessionRef, and tce-Id in VarLogMeasReport;

1>	store the received bt-NameList, if included, in VarLogMeasConfig;

1>	store the received wlan-Namelist, if included, in VarLogMeasConfig;

1>	store the received sensor-Namelist, if included, in VarLogMeasConfig;

1>	start timer T330 with the timer value set to the loggingDuration;

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if the logMeasReportReq is present and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3>	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3>	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3>	include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3>	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3>	include the logMeasInfoList and set it to include one or more entries from VarLogMeasReport starting from the entries logged first;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

3>	if the VarLogMeasReport includes one or more additional logged Bluetooth measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailableBT;

3>	if the VarLogMeasReport includes one or more additional logged WLAN measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailableWLAN;

…

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1.

8.1.6.1.2.10.3	Test description

8.1.6.1.2.10.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1, NR Cell 3, NR Cell 11

-	System information combination NR-4 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

-	When NR Cell 3 is used its PLMN will be set to PLMN2.

UE:

-	None.

Preamble:

-	The UE is in state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3 on NR Cell 1.

8.1.6.1.2.10.3.2	Test procedure sequence

Table 8.1.6.1.2.10.3.2-1/2 illustrate the downlink power levels and other changing parameters to be applied for the cell at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.6.1.2.10.3.2-1: Time instances of cell power level for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 11 | NR Cell 3 |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -85 | Off | Off |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | Off | Off | -85 |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | Off | -85 | Off |
| Note: The downlink signal level uncertainty is specified in TS 38.508-1 [4] clause 6.2.2.1. |  |  |  |  |  |

Table 8.1.6.1.2.10.3.2-2: Time instances of cell power level for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 11 | NR Cell 3 |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -82 | Off | Off |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | Off | Off | -82 |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | Off | -82 | Off |
| Note: The downlink signal level uncertainty is specified in TS 38.508-1 [4] clause 6.2.2.2.2. |  |  |  |  |  |

Table 8.1.6.1.2.10.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a LoggedMeasurementConfiguration message on NR Cell 1. | <-- | NR RRC: LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits a RRCRelease message to release the RRC connection. | <-- | NR RRC: RRCRelease | - | - |
| 3 | Wait 10 seconds for UE performing the logging at regular time intervals. | - | - | - | - |
| 4 | The SS changes NR Cell 1 and NR Cell 3 level according to the row "T1" in Table 8.1.6.1.2.10.3.2-1/2. | - | - | - | - |
| 5-10 | Steps 1 to 6a1 of generic test procedure in TS 38.508-1 [4] subclause 4.9.5.2.2-1 are performed on NR Cell 3. | - | - | - | - |
| 11 | Wait 10 seconds for UE not to perform the logging at regular time intervals. | - | - | - | - |
| 12 | The SS transmits a Paging message. | <-- | NR RRC: Paging | - | - |
| 13 | The UE transmits an RRCRequest message. | --> | NR RRC: RRCRequest | - | - |
| 14 | The SS transmits an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 15 | Check: Does UE transmit an RRCSetupComplete message with logMeasAvailable IE not present? | --> | NR RRC: RRCSetupComplete | 1 | P |
| 15A-15D | Steps 5 to 8 of the generic procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 16 | The SS transmits a RRCRelease message to release RRC connection and move to RRC_IDLE on NR Cell 3. | <-- | NR RRC: RRCRelease | - | - |
| 17 | The SS changes NR Cell 1, NR Cell 11 and NR Cell 3 level according to the row "T2" in Table 8.1.6.1.2.10.3.2-1/2. | - | - | - | - |
| 18-23 | Steps 1 to 6a1 of generic test procedure in TS 38.508-1 [4] subclause 4.9.5.2.2-1 are performed on NR Cell 11. | - | - | - | - |
| 24 | Wait 10 seconds for UE performing the logging at regular time intervals. | - | - | - | - |
| 25 | The SS transmits a Paging message. | <-- | NR RRC: Paging | - | - |
| 26 | The UE transmits an RRCRequest message. | --> | NR RRC: RRCRequest | - | - |
| 27 | The SS transmits an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 28 | Check: Does UE transmit an RRCSetupComplete message with logMeasAvailable set as true? | --> | NR RRC: RRCSetupComplete | 3 | P |
| 29-31 | Steps 5 to 8 of the generic procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 32 | The SS send a UEInformationRequest message to get logMeasReport. | <-- | NR RRC: UEInformationRequest | - | - |
| 33 | Check: Does the UE send an UEInformationResponse message including at least one logMeasReport with serving cell measurements for NR Cell 1 and NR Cell 11 and without serving cell measurements for NR Cell 3? | --> | NR RRC: UEInformationResponse | 2, 4, | P |

8.1.6.1.2.10.3.3	Specific message contents

Table 8.1.6.1.2.10.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.1.6.1.2.10.3.2-3)

Derivation path: TS 38.508-1 [4], table 4.6.1-5AA with condition PERIODICAL

Table 8.1.6.1.2.10.3.3-1A: RRCSetupComplete (step 15, Table 8.1.6.1.2.10.3.2-3)

| Derivation path: TS 38.508-1 [4], table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { | Checked | Note 1 |  |
| ue-MeasurementsAvailable-r16 SEQUENCE { |  |  | If nonCriticalExtension is present |
| logMeasAvailable-r16 | Not present |  | If ue-MeasurementsAvailable-r16 is present |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| If nonCriticalExtension and ue-MeasurementsAvailable-r16 arepresent | Contents of ue-MeasurementsAvailable-r16 shall be checked only if nonCriticalExtension and ue-MeasurementsAvailable-r16 are present |
| Note 1: If the UE has no underlying information to report to the network, this information element may be absent. If this information element and MeasurementsAvailable-r16 are present, further check on the contents of the ue-MeasurementsAvailable-r16 shall be performed |  |

Table 8.1.6.1.2.10.3.3-2: RRCSetupComplete (step 28, Table 8.1.6.1.2.10.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ue-MeasurementsAvailable-r16 | UE-MeasurementsAvailable-r16 with condition LOG |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.2.10.3.3-3: UEInformationRequest (step 32, Table 8.1.6.1.2.10.3.2-3)

Derivation path: TS 38.508-1 [4], Table 4.6.1-32A with condition LOG

Table 8.1.6.1.2.10.3.3-4: UEInformationResponse (step 33, Table 8.1.6.1.2.10.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| logMeasReport-r16 SEQUENCE { |  |  |  |
| absoluteTimeStamp-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceReference-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 SEQUENCE { |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |
| traceId-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |
| traceRecordingSessionRef-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| tce-Id-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r16 SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-r16 SEQUENCE { | At least 2 entries where at least one entry complies to entry with index ‘x’ below. At least 1 entry with “servCellIdentity-r16” set to NR Cell 1 and at least 1 entry with “servCellIdentity-r16” set to NR Cell 11. |  |  |
| LogMeasInfo-r16[x] SEQUENCE { |  | entry x |  |
| locationInfo-r16 | Not checked |  |  |
| relativeTimeStamp-r16 | SS record the value |  |  |
| servCellIdentity-r16 | Same as NR Cell 1 or NR Cell 11 |  |  |
| measResultServCell-16 SEQUENCE { |  |  |  |
| resultsSSB-Cell-r16 | MeasQuantityResults of NR Cell 1 or NR Cell 11 |  |  |
| resultsSSB SEQUENCE { |  |  |  |
| best-ssb-Index | Not checked |  |  |
| best-ssb-Results | Not checked |  |  |
| numberOfGoodSSB | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells-r16 | Any allowed value |  |  |
| anyCellSelectionDetected-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| logMeasAvailable-r16 | Not present |  |  |
| logMeasAvailableBT-r16 | Not present |  |  |
| logMeasAvailableWLAN-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

###### 8.1.6.1.2.11	Logged MDT / UE state transitions

8.1.6.1.2.11.1	Test Purpose (TP)

(1)

with { UE received LoggedMeasurementConfiguration message and has stored logMeasReport in VarLogMeasReport }

ensure that {

when { UE moves to RRC_CONNECTED state }

then { UE maintains the Logged measurement configurations and logged measurement reports }

}

(2)

Void

(3)

with { UE received LoggedMeasurementConfiguration message and is not in ‘camped normally’ state }

ensure that {

when { UE is again in ‘camped normally’ state on an NR cell }

then { UE performs logged MDT measurements }

}

8.1.6.1.2.11.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clauses 5.3.5.3, 5.5.4.1, 5.5.4.2 and 5.5.5. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.3.3.4]

The UE shall perform the following actions upon reception of the RRCSetup:

…

1>	set the content of RRCSetupComplete message as follows:

…

2>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable in the RRCSetupComplete message;

2>	if the UE has Bluetooth logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailableBT in the RRCSetupComplete message;

2>	if the UE has WLAN logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailableWLAN in the RRCSetupComplete message;

…

[TS 38.331, clause 5.5a.1.3]

Upon receiving the LoggedMeasurementConfiguration message the UE shall:

1>	discard the logged measurement configuration as well as the logged measurement information as specified in 5.5a.2;

1>	store the received loggingDuration, reportType and areaConfiguration, if included, in VarLogMeasConfig;

1>	if the LoggedMeasurementConfiguration message includes plmn-IdentityList:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN as well as the PLMNs included in plmn-IdentityList;

1>	else:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN;

1>	store the received absoluteTimeInfo, traceReference, traceRecordingSessionRef, and tce-Id in VarLogMeasReport;

1>	store the received bt-NameList, if included, in VarLogMeasConfig;

1>	store the received wlan-Namelist, if included, in VarLogMeasConfig;

1>	store the received sensor-Namelist, if included, in VarLogMeasConfig;

1>	start timer T330 with the timer value set to the loggingDuration;

[TS 38.331, clause5.5a.3.2]

While T330 is running, the UE shall:

1>	perform the logging in accordance with the following:

2>	if the reportType is set to periodical in the VarLogMeasConfig:

3>	if the UE is in any cell selection state (as specified in TS 38.304 [20]):

4>	perform the logging at regular time intervals, as defined by the loggingInterval in the VarLogMeasConfig;

3>	if the UE is in camped normally state on an NR cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

4>	if areaConfiguration is not included in VarLogMeasConfig; or

4>	if the serving cell is part of the area indicated by areaConfig in areaConfiguration in VarLogMeasConfig:

5>	perform the logging at regular time intervals, as defined by the loggingInterval in the VarLogMeasConfig;

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if the logMeasReportReq is present and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3>	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3>	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3>	include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3>	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3>	include the logMeasInfoList and set it to include one or more entries from VarLogMeasReport starting from the entries logged first;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

3>	if the VarLogMeasReport includes one or more additional logged Bluetooth measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailableBT;

3>	if the VarLogMeasReport includes one or more additional logged WLAN measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailableWLAN;

…

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1.

8.1.6.1.2.11.3	Test description

8.1.6.1.2.11.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 and NR Cell 11.

-	System information combination NR-2 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

-	None.

Preamble:

-	The UE is in state 3N-A on NR Cell 1 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

8.1.6.1.2.11.3.2	Test procedure sequence

Table 8.1.6.1.2.11.3.2-1/2 illustrate the downlink power levels and other changing parameters to be applied for the cell at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.6.1.2.11.3.2-1: Time instances of cell power level for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 11 |
| --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -85 | Off |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | Off | Off |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | Off | -85 |

Table 8.1.6.1.2.11.3.2-2: Time instances of cell power level for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 11 |
| --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -82 | Off |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | Off | Off |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | Off | -82 |

Table 8.1.6.1.2.11.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits a LoggedMeasurementConfiguration message including to configure the UE to perform logging of measurement results while in RRC_IDLE. | <-- | NR RRC: LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits an RRCRelease message to move the UE to RRC_IDLE state. | <-- | NR RRC: RRCRelease | - | - |
| 3 | Wait 10s to allow UE to activate logging. | - | - | - | - |
| 4 | The SS changes NR Cell 1 and NR Cell 11 levels according to the row "T1" in table 8.1.6.1.2.11.3.2-1/2. | - | - | - | - |
| 5 | Wait 15 seconds. | - | - | - | - |
| 6 | The SS changes NR Cell 1 and NR Cell 11 levels according to the row "T2" in table 8.1.6.1.2.11.3.2-1/2. | - | - | - | - |
| 7-11 | Steps 1 to 5 of generic test procedure in TS 38.508-1 [4] subclause 4.9.5.2.2-1 are performed on NR Cell 11. | - | - | - | - |
| 12 | Wait 8 seconds. Note: It is checked that UE does not perform the logging while in RRC_CONNECTED state. | - | - | - | - |
| 13 | The SS transmits an RRCRelease message to move the UE to RRC_IDLEstate. | <-- | NR RRC: RRCRelease | - | - |
| 14 | Wait 10s to allow UE to activate logging. | - | - | - | - |
| 15-22 | Steps 1 to 8 of the generic procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure on NR Cell 11. | - | - | - | - |
| 23 | The SS transmits a UEInformationRequest message on NR Cell 11. | <-- | NR RRC: UEInformationRequest | - | - |
| 24 | Check: Does the UE transmit a UEInformationResponse message including logMeasReport with serving cell measurements for NR Cell 1 and NR Cell 11? | --> | NR RRC: UEInformationResponse | 1, 3 | P |

8.1.6.1.2.11.3.3	Specific message contents

Table 8.1.6.1.2.11.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.1.6.1.2.11.3.2-3)

Derivation path: TS 38.508-1 [4], Table 4.6.1-5AA with condition PERIODICAL

Table 8.1.6.1.2.11.3.3-2: UEInformationRequest (step 23, Table 8.1.6.1.2.11.3.2-3)

Derivation path: TS 38.508-1 [4], Table 4.6.1-32A with condition LOG

Table 8.1.6.1.2.11.3.3-3: UEInformationResponse (step 24, Table 8.1.6.1.2.11.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| logMeasReport-r16 SEQUENCE { |  |  |  |
| absoluteTimeStamp-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceReference-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 SEQUENCE { |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |
| traceId-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |
| traceRecordingSessionRef-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| tce-Id-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r16 SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-r16 SEQUENCE { | At least 2 entries, Where at least 1 entry with servCellIdentity-r16 set to cell Identity of NR Cell 1 and at least 1 entry with servCellIdentity-r16 set to cell Identity of NR Cell 11. |  |  |
| LogMeasInfo-r16[x] SEQUENCE { |  | entry x |  |
| locationInfo-r16 | Not checked |  |  |
| relativeTimeStamp-r16 | SS record the value |  |  |
| servCellIdentity-r16 | Same as NR Cell 1 or NR Cell 11 |  |  |
| measResultServCell-16 SEQUENCE { |  |  |  |
| resultsSSB-Cell-r16 | MeasQuantityResults of NR Cell 1 or NR Cell 11 |  |  |
| resultsSSB SEQUENCE { |  |  |  |
| best-ssb-Index | Not checked |  |  |
| best-ssb-Results | Not checked |  |  |
| numberOfGoodSSB | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells-r16 | Any allowed value |  |  |
| anyCellSelectionDetected-r16 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| logMeasAvailable-r16 | Not present |  |  |
| logMeasAvailableBT-r16 | Not present |  |  |
| logMeasAvailableWLAN-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

###### 8.1.6.1.2.12	Logged MDT / Release of logged MDT measurement configuration / Expire of duration timer

8.1.6.1.2.12.1	Test Purpose (TP)

(1)

with { UE received LoggedMeasurementConfiguration message and has stored logMeasReport in VarLogMeasReport }

ensure that {
  when { The logging duration timer T330 expires }

then { UE releases VarLogMeasConfig and will not perform logged measurements }

}

(2)

with { UE received LoggedMeasurementConfiguration message and has stored logMeasReport in VarLogMeasReport }

ensure that {

when { UE has released VarlogMeasConfig due to timer T330 has expired and UE receives UEInformationRequest message with logMeasReportReq present }

then { UE transmits an UEInformationResponse message including logMeasReport }

}

(3)

with { UE received LoggedMeasurementConfiguration message and has stored logMeasReport in VarLogMeasReport }

ensure that {

when { UE has transmitted all the stored logMeasReport and UE receives UEInformationRequest message with logMeasReportReq present }

then { UE transmits an UEInformationResponse message without logMeasReport }

}

8.1.6.1.2.12.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clauses 5.3.3.4, 5 5.5a.1.3, 5.5a.1.4, 5.5a.3.2, and 5.7.10.3. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.3.3.4]

The UE shall perform the following actions upon reception of the RRCSetup:

…

2>	set the content of RRCSetupComplete message as follows:

…

2>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable in the RRCSetupComplete message;

2>	if the UE has Bluetooth logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailableBT in the RRCSetupComplete message;

2>	if the UE has WLAN logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailableWLAN in the RRCSetupComplete message;

…

[TS 38.331, clause 5.5a.1.3]

Upon receiving the LoggedMeasurementConfiguration message the UE shall:

1>	discard the logged measurement configuration as well as the logged measurement information as specified in 5.5a.2;

1>	store the received loggingDuration, reportType and areaConfiguration, if included, in VarLogMeasConfig;

1>	if the LoggedMeasurementConfiguration message includes plmn-IdentityList:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN as well as the PLMNs included in plmn-IdentityList;

1>	else:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN;

1>	store the received absoluteTimeInfo, traceReference, traceRecordingSessionRef, and tce-Id in VarLogMeasReport;

1>	store the received bt-NameList, if included, in VarLogMeasConfig;

1>	store the received wlan-Namelist, if included, in VarLogMeasConfig;

1>	store the received sensor-Namelist, if included, in VarLogMeasConfig;

1>	start timer T330 with the timer value set to the loggingDuration;

[TS 38.331, clause5.5a.3.2]

While T330 is running, the UE shall:

1>	perform the logging in accordance with the following:

2>	if the reportType is set to periodical in the VarLogMeasConfig:

3>	if the UE is camping normally on an NR cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and, if the cell is part of the area indicated by areaConfiguration if configured in VarLogMeasConfig:

4>	perform the logging at regular time intervals, as defined by the loggingInterval in the VarLogMeasConfig;

2>	else if the reportType is set to eventTriggered, and eventType is set to outOfCoverage:

3>	perform the logging at regular time intervals as defined by the loggingInterval in VarLogMeasConfig only when the UE is in any cell selection state;

3>	perform the logging immediately upon transitioning from the any cell selection state to the camped normally state;

2>	else if the reportType is set to eventTriggered and eventType is set to eventL1:

3>	if the UE is camping normally on an NR cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and, if the cell is part of the area indicated by areaConfiguration if configured in VarLogMeasConfig;

4>	perform the logging at regular time intervals as defined by the loggingInterval in VarLogMeasConfig only when the conditions indicated by the eventL1 are met;

2>	when performing the logging:

3>	set the relativeTimeStamp to indicate the elapsed time since the moment at which the logged measurement configuration was received;

3>	if detailed location information became available during the last logging interval, set the content of the locationInfo as in 5.3.3.7:

3>	if the UE is in any cell selection state (as specified in TS 38.304 [20]):

4>	set anyCellSelectionDetected to indicate the detection of no suitable or no acceptable cell found;

4>	set the servCellIdentity to indicate global cell identity of the last logged cell that the UE was camping on;

4>	set the measResultServingCell to include the quantities of the last logged cell the UE was camping on;

3>	else:

4>	set the servCellIdentity to indicate global cell identity of the cell the UE is camping on;

4>	set the measResultServingCell to include the quantities of the cell the UE is camping on;

4>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements that became available during the last logging interval for at most the following number of neighbouring cells: 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies per RAT and according to the following:

5>	for each neighbour cell included, include the optional fields that are available;

NOTE:	The UE includes the latest results of the available measurements as used for cell reselection evaluation in RRC_IDLE or RRC_INACTIVE, which are performed in accordance with the performance requirements as specified in TS 38.133 [14].

2>	when the memory reserved for the logged measurement information becomes full, stop timer T330 and perform the same actions as performed upon expiry of T330, as specified in 5.5a.1.4.

[TS 38.331, clause 5.5a.1.4]

Upon expiry of T330 the UE shall:

1>	release VarLogMeasConfig;

The UE is allowed to discard stored logged measurements, i.e. to release VarLogMeasReport, 48 hours after T330 expiry.

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if the logMeasReportReq is present and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3>	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3>	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3>	include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3>	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3>	include the logMeasInfoList and set it to include one or more entries from VarLogMeasReport starting from the entries logged first;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

3>	if the VarLogMeasReport includes one or more additional logged Bluetooth measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailableBT;

3>	if the VarLogMeasReport includes one or more additional logged WLAN measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailableWLAN;

…

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1.

8.1.6.1.2.12.3	Test description

8.1.6.1.2.12.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

UE:

-	None.

Preamble:

-	The UE is in state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

8.1.6.1.2.12.3.2	Test procedure sequence

Table 8.1.6.1.2.12.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a LoggedMeasurementConfiguration message on NR Cell 1. | <-- | NR RRC: LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits a RRCRelease message to release the RRC connection. | <-- | NR RRC: RRCRelease | - | - |
| 3 | Wait 30 seconds for the UE to perform the logging at regular time intervals. | - | - | - | - |
| 4-11 | Steps 1 to 8 of the generic procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 12 | Void | - | - | - | - |
| 13 | The SS sends an UEInformationRequest message to get logMeasReport. | <-- | NR RRC: UEInformationRequest | - | - |
| 14 | The UE sends an UEInformationResponse message on SRB2 including logMeasReport. | --> | NR RRC: UEInformationResponse | - | - |
| - | EXCEPTION: In case the logMeasAvailable in logMeasReport is True, steps 14a1 and 14a2 will be executed. | - | - | - | - |
| 14a1 | The SS sends an UEInformationRequest message to get logMeasReport. | <-- | NR RRC: UEInformationRequest | - | - |
| 14a2 | The UE sends an UEInformationResponse message on SRB2 including logMeasReport. | --> | NR RRC: UEInformationResponse | - | - |
| 15 | The SS sends an UEInformationRequest message to get logMeasReport. | <-- | NR RRC: UEInformationRequest | - | - |
| 16 | Check: Does the UE send an UEInformationResponse message on SRB1 without logMeasReport? | --> | NR RRC: UEInformationResponse | 3 | P |
| 17 | The SS transmits a RRCRelease message to release the RRC connection. | <-- | NR RRC: RRCRelease | - | - |
| 18 | Wait 10 minutes for UE to perform the logging at regular time intervals as to ensure timer T330 has expired. | - | - | - | - |
| 19-26 | Steps 1 to 8 of the generic procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 27 | Void | - | - | - | - |
| 28 | The SS sends an UEInformationRequest message to get logMeasReport. | <-- | NR RRC: UEInformationRequest | - | - |
| 29 | Check: Does the UE send an UEInformationResponse message on SRB2 including logMeasReport? | --> | NR RRC: UEInformationResponse | 2 | P |
| - | EXCEPTION: In case the logMeasAvailable in logMeasReport is True, steps 29a1 and 29a2 will be executed until all the stored logMeasReport transmitted. | - | - | - | - |
| 29a1 | The SS sends an UEInformationRequest message to get logMeasReport. | <-- | NR RRC: UEInformationRequest | - | - |
| 29a2 | UE sends an UEInformationResponse message on SRB2 including logMeasReport. | --> | NR RRC: UEInformationResponse | - | - |
| 30 | The SS transmits a RRCRelease message to release the RRC connection. | <-- | NR RRC: RRCRelease | - | - |
| 31 | Wait 30 seconds. | - | - | - | - |
| 32 | The SS transmits a Paging message on NR Cell 1. | <-- | NR RRC: Paging | - | - |
| 33 | The UE transmits an RRCRequest message on NR Cell 1. | --> | NR RRC: RRCRequest | - | - |
| 34 | The SS transmits an RRCSetup message on NR Cell 1. | <-- | NR RRC: RRCSetup | - | - |
| 35 | Check: Does UE transmit an RRCSetupComplete message with logMeasAvailable IE not present? | --> | NR RRC: RRCSetupComplete | 1 | P |
| 36-39 | Steps 5 to 8 of the generic procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure. | - | - | - | - |

8.1.6.1.2.12.3.3	Specific message contents

Table 8.1.6.1.2.12.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.1.6.1.2.12.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-5AA with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| loggedMeasurementConfiguration-r16 SEQUENCE { |  |  |  |
| loggingDuration-r16 | min10 | 10 minutes for T330 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.2.12.3.3-2: RRCSetupComplete (step 35, Table 8.1.6.1.2.12.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { | Checked | Note 1 |  |
| ue-MeasurementsAvailable-r16 SEQUENCE { |  |  | If nonCriticalExtension is present |
| logMeasAvailable-r16 | Not present |  | If ue-MeasurementsAvailable-r16 is present |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| If nonCriticalExtension and ue-MeasurementsAvailable-r16 are present | Contents of ue-MeasurementsAvailable-r16 shall be checked only if nonCriticalExtension and ue-MeasurementsAvailable-r16 are present |
| Note 1: If the UE has no underlying information to report to the network, this information element may be absent. If this information element is present, futher check on the contents of the ue-MeasurementsAvailable-r16 shall be performed. |  |

Table 8.1.6.1.2.12.3.3-3: UEInformationRequest (steps 13, 14a1, 15, 28 and 29a1, Table 8.1.6.1.2.12.3.2-1)

Derivation path: TS 38.508-1 [4], Table 4.6.1-32A with condition LOG

Table 8.1.6.1.2.12.3.3-4: UEInformationResponse (steps 14, 14a2, 29 and 29a2, Table 8.1.6.1.2.12.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| logMeasReport-r16 SEQUENCE { |  |  |  |
| absoluteTimeStamp-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceReference-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 SEQUENCE { |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |
| traceId-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |
| traceRecordingSessionRef-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| tce-Id-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r16 SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-r16 SEQUENCE { | At least 2 entries where at least one entry complies to entry with index ‘x’ below. SS records the relativeTimeStamp value for each entry |  |  |
| LogMeasInfo-r16[x] SEQUENCE { |  | entry x |  |
| locationInfo-r16 | Not checked |  |  |
| relativeTimeStamp-r16 | SS record the value |  |  |
| servCellIdentity-r16 | Same as NR Cell 1 |  |  |
| measResultServCell-16 SEQUENCE { |  |  |  |
| resultsSSB-Cell-r16 | MeasQuantityResults of NR Cell 1 |  |  |
| resultsSSB SEQUENCE { |  |  |  |
| best-ssb-Index | Not checked |  |  |
| best-ssb-Results | Not checked |  |  |
| numberOfGoodSSB | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells-r16 | Any allowed value |  |  |
| anyCellSelectionDetected-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| logMeasAvailable-r16 | Not checked |  |  |
| logMeasAvailableBT-r16 | Not present |  |  |
| logMeasAvailableWLAN-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.2.12.3.3-5: UEInformationResponse (step 16, Table 8.1.6.1.2.12.3.2-1)

| Derivation path: TS 38.508-1 [4] Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| logMeasReport-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.2.12.3.3-6: RRCSetupComplete (steps 7 and 22, Table 8.1.6.1.2.12.3.2-1)

| Derivation path: TS 38.508-1 [4] Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ue-MeasurementsAvailable-r16 | UE-MeasurementsAvailable-r16 with condition LOG |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

###### 8.1.6.1.2.13	Logged MDT / Release of logged MDT measurement configuration / Reception of new logged measurement configuration

8.1.6.1.2.13.1	Test Purpose (TP)

(1)

with { UE received LoggedMeasurementConfiguration message and has stored logMeasReport in VarLogMeasReport }

ensure that {

when { UE receive a new LoggedMeasurementConfiguration message }

then { UE discards the stored logMeasReport }

}

(2)

with { UE received LoggedMeasurementConfiguration message and has stored logMeasReport in VarLogMeasReport }

ensure that {

when { UE receive a new LoggedMeasurementConfiguration message }

then { UE discards the old logged measurement configuration }

}

8.1.6.1.2.13.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clauses 5.3.3.4, 5 5.5a.1.3 and 5.5a.3.2. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.3.3.4]

The UE shall perform the following actions upon reception of the RRCSetup:

…

3>	set the content of RRCSetupComplete message as follows:

…

2>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable in the RRCSetupComplete message;

2>	if the UE has Bluetooth logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailableBT in the RRCSetupComplete message;

2>	if the UE has WLAN logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailableWLAN in the RRCSetupComplete message;

…

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if the logMeasReportReq is present and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3>	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3>	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3>	include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3>	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3>	include the logMeasInfoList and set it to include one or more entries from VarLogMeasReport starting from the entries logged first;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

3>	if the VarLogMeasReport includes one or more additional logged Bluetooth measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailableBT;

3>	if the VarLogMeasReport includes one or more additional logged WLAN measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailableWLAN;

…

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1.

[TS 38.331, clause 5.5a.1.3]

Upon receiving the LoggedMeasurementConfiguration message the UE shall:

1>	discard the logged measurement configuration as well as the logged measurement information as specified in 5.5a.2;

1>	store the received loggingDuration, reportType and areaConfiguration, if included, in VarLogMeasConfig;

1>	if the LoggedMeasurementConfiguration message includes plmn-IdentityList:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN as well as the PLMNs included in plmn-IdentityList;

1>	else:

2>	set plmn-IdentityList in VarLogMeasReport to include the RPLMN;

1>	store the received absoluteTimeInfo, traceReference, traceRecordingSessionRef, and tce-Id in VarLogMeasReport;

1>	store the received bt-NameList, if included, in VarLogMeasConfig;

1>	store the received wlan-Namelist, if included, in VarLogMeasConfig;

1>	store the received sensor-Namelist, if included, in VarLogMeasConfig;

1>	start timer T330 with the timer value set to the loggingDuration;

[TS 38.331, clause5.5a.3.2]

While T330 is running, the UE shall:

1>	perform the logging in accordance with the following:

2>	if the reportType is set to periodical in the VarLogMeasConfig:

3>	if the UE is camping normally on an NR cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and, if the cell is part of the area indicated by areaConfiguration if configured in VarLogMeasConfig:

4>	perform the logging at regular time intervals, as defined by the loggingInterval in the VarLogMeasConfig;

2>	else if the reportType is set to eventTriggered, and eventType is set to outOfCoverage:

3>	perform the logging at regular time intervals as defined by the loggingInterval in VarLogMeasConfig only when the UE is in any cell selection state;

3>	perform the logging immediately upon transitioning from the any cell selection state to the camped normally state;

2>	else if the reportType is set to eventTriggered and eventType is set to eventL1:

3>	if the UE is camping normally on an NR cell and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport and, if the cell is part of the area indicated by areaConfiguration if configured in VarLogMeasConfig;

4>	perform the logging at regular time intervals as defined by the loggingInterval in VarLogMeasConfig only when the conditions indicated by the eventL1 are met;

2>	when performing the logging:

3>	set the relativeTimeStamp to indicate the elapsed time since the moment at which the logged measurement configuration was received;

3>	if detailed location information became available during the last logging interval, set the content of the locationInfo as in 5.3.3.7:

3>	if the UE is in any cell selection state (as specified in TS 38.304 [20]):

4>	set anyCellSelectionDetected to indicate the detection of no suitable or no acceptable cell found;

4>	set the servCellIdentity to indicate global cell identity of the last logged cell that the UE was camping on;

4>	set the measResultServingCell to include the quantities of the last logged cell the UE was camping on;

3>	else:

4>	set the servCellIdentity to indicate global cell identity of the cell the UE is camping on;

4>	set the measResultServingCell to include the quantities of the cell the UE is camping on;

4>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements that became available during the last logging interval for at most the following number of neighbouring cells: 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies per RAT and according to the following:

5>	for each neighbour cell included, include the optional fields that are available;

NOTE:	The UE includes the latest results of the available measurements as used for cell reselection evaluation in RRC_IDLE or RRC_INACTIVE, which are performed in accordance with the performance requirements as specified in TS 38.133 [14].

2>	when the memory reserved for the logged measurement information becomes full, stop timer T330 and perform the same actions as performed upon expiry of T330, as specified in 5.5a.1.4.

8.1.6.1.2.13.3	Test description

8.1.6.1.2.13.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

UE:

-	None.

Preamble:

-	The UE is in state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

8.1.6.1.2.13.3.2	Test procedure sequence

Table 8.1.6.1.2.13.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a LoggedMeasurementConfiguration message on NR Cell 1. | <-- | NR RRC: LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits a RRCRelease message to release the RRC connection. | <-- | NR RRC: RRCRelease | - | - |
| 3 | Wait 30 seconds for the UE to perform the logging at regular time intervals. | - | - | - | - |
| 4-11 | Steps 1 to 8 of the generic procedure in TS 38.508-1 [4], Table 4.5.4.2-3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 12 | Void | - | - | - | - |
| 13 | The SS transmits a LoggedMeasurementConfiguration message on NR Cell 1. | <-- | NR RRC: LoggedMeasurementConfiguration | - | - |
| 14 | The SS sends an UEInformationRequest message to get logMeasReport. | <-- | NR RRC: UEInformationRequest | - | - |
| 15 | Check: Does the UE send an UEInformationResponse message on SRB1 without logMeasReport? | --> | NR RRC: UEInformationResponse | 1 | P |
| 16 | The SS transmits a RRCRelease message to release the RRC connection. | <-- | NR RRC: RRCRelease | - | - |
| 17 | Wait 30 seconds for the UE to perform the logging at regular time intervals. | - | - | - | - |
| 18-25 | Steps 1 to 8 of the generic procedure in TS 38.508-1 [4], Table 4.5.4.2-3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 26 | Void | - | - | - | - |
| 27 | The SS sends an UEInformationRequest message to get logMeasReport. | <-- | NR RRC: UEInformationRequest | - | - |
| 28 | Check: Does the UE send an UEInformationResponse message on SRB2 without logMeasReport based on the LoggedMeasurementConfiguration as in step 1, but with logMeasReport based on the LoggedMeasurementConfiguration as in step 13? | --> | NR RRC: UEInformationResponse | 2 | P |

8.1.6.1.2.13.3.3	Specific message contents

Table 8.1.6.1.2.13.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.1.6.1.2.13.3.2-1)

Derivation path: TS 38.508-1 [4], Table 4.6.1-5AA with condition PERIODICAL

Table 8.1.6.1.2.13.3.3-1A: LoggedMeasurementConfiguration (step 13, Table 8.1.6.1.2.13.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-5AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| loggedMeasurementConfiguration-r16 SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| loggingInterval-r16 | ms5120 | 5.12 seconds |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.2.13.3.3-2: RRCSetupComplete (steps 7 and 21, Table 8.1.6.1.2.13.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ue-MeasurementsAvailable-r16 | UE-MeasurementsAvailable-r16 with condition LOG |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.2.13.3.3-3: UEInformationRequest (steps 14 and 27, Table 8.1.6.1.2.13.3.2-1)

Derivation path: TS 38.508-1 [4], Table 4.6.1-32A with condition LOG

Table 8.1.6.1.2.13.3.3-4: UEInformationResponse (step 15, Table 8.1.6.1.2.13.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| logMeasReport-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.2.13.3.3-5: UEInformationResponse (step 28, Table 8.1.6.1.2.13.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| logMeasReport-r16 SEQUENCE { |  |  |  |
| absoluteTimeStamp-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 13 |  |  |
| traceReference-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 SEQUENCE { |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit | Same value as sent by SS in LoggedMeasurementConfiguration in step 13 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit | Same value as sent by SS in LoggedMeasurementConfiguration in step 13 |  |  |
| } |  |  |  |
| traceId-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 13 |  |  |
| } |  |  |  |
| traceRecordingSessionRef-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 13 |  |  |
| tce-Id-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 13 |  |  |
| logMeasInfoList-r16 SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-r16 SEQUENCE { | At least 1 entries where at least one entry complies to entry with index ‘x’ below. SS records the relativeTimeStamp value for each entry |  |  |
| LogMeasInfo-r16[x] SEQUENCE { |  | entry x |  |
| locationInfo-r16 | Not checked |  |  |
| relativeTimeStamp-r16 | SS record the value |  |  |
| servCellIdentity-r16 | Same as NR Cell 1 |  |  |
| measResultServCell-16 SEQUENCE { |  |  |  |
| resultsSSB-Cell-r16 | MeasQuantityResults of NR Cell 1 |  |  |
| resultsSSB SEQUENCE { |  |  |  |
| best-ssb-Index | Not checked |  |  |
| best-ssb-Results | Not checked |  |  |
| numberOfGoodSSB | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells-r16 | Any allowed value |  |  |
| anyCellSelectionDetected-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| logMeasAvailable-r16 | Not present |  |  |
| logMeasAvailableBT-r16 | Not present |  |  |
| logMeasAvailableWLAN-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

###### 8.1.6.1.2.14	Logged MDT / RRC_IDLE / Logging and reporting / IDC mechanism

8.1.6.1.2.14.1	Test Purpose (TP)

(1)

with { UE is performing measurement logging and T330 is running }

ensure that {

when { UE detects IDC problem }

then { UE suspend measurement logging and tag MDT report with inDeviceCoexDetected-r17 flag in logMeasInfoList-r16 in VarLogMeasReport }

}

(2)

with { UE has suspended measurement logging and T330 is running }

ensure that {

when { The IDC problems detected by the UE is resolved during the last logging interval }

then { UE resume measurement logging }

}

8.1.6.1.2.14.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clauses 5.5a.3.2. Unless otherwise stated these are Rel-17 requirements.

[TS 38.331 clause 5.5a.3.2]

While T330 is running and SDT procedure is not ongoing, the UE shall:

1>	if measurement logging is suspended:

2>	if during the last logging interval the IDC problems detected by the UE is resolved, resume measurement logging;

1>	if not suspended, perform the logging in accordance with the following:

...

2>	when performing the logging:

3> if InterFreqTargetInfo is configured and if the UE detected IDC problems on at least one of the frequencies included in InterFreqTargetInfo during the last logging interval, or

3>	if InterFreqTargetInfo is not configured and if the UE detected IDC problems during the last logging interval:

4>	if measResultServingCell in the VarLogMeasReport is not empty:

5>	include inDeviceCoexDetected;

5>	suspend measurement logging from the next logging interval;

4>	else:

5>	suspend measurement logging;

8.1.6.1.2.14.3	Test description

8.1.6.1.2.14.3.1	Pre-test conditions

System Simulator:

-	For operating band n40, NR Cell 14/NRf4 (2390-2400MHz) and WLAN AP1 (Cell 27) as specified in cl. 4.13 TS 38.508-1[4] are used;

-	For operating band n41, NR Cell 1/NRf1 (2496-2556MHz) and WLAN AP2 (Cell 27b) as specified in cl. 4.13 TS 38.508-1[4] are used;

UE:

UE’s WLAN is on;

Preamble:

The UE is in state 3N-A on NR Cell 14/NR Cell 1 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

8.1.6.1.2.14.3.2	Test procedure sequence

Table 8.1.6.1.2.14.3.2-1a for FR1 illustrate the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. The configuration “C1” for "T0" indicates the initial conditions. Subsequent configurations marked "T1" and "T2" are applied at the points indicated in the Main behaviour description in Table 8.1.6.1.2.14.3.2-1. The configuration “C1” for “T0” and “C2” for “T1” are specified in TS 38.508-1 [4] Table 4.13-1.

Table 8.1.6.1.2.14.3.2-1a: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 14 /NR Cell 1 | WLAN AP | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Refer to “C1” in TS 38.508-1 [4] |  |  |  |  |
| T1 | Refer to “C2” in TS 38.508-1 [4] |  |  |  |  |
| T2 | Refer to “C1” in TS 38.508-1 [4] |  |  |  |  |

Table 8.1.6.1.2.14.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a LoggedMeasurementConfiguration message on NR Cell 14/NR Cell 1. | <-- | NR RRC: LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits a RRCRelease message to release the RRC connection. | <-- | NR RRC: RRCRelease | - | - |
| 3 | Wait 3.5s for UE to activate logging. | - | - | - | - |
| 3A | The SS re-adjusts the power levels according to row "T1" in Table 8.1.6.1.2.14.3.2-1a. | - | - | - | - |
| 3B | Void | - | - | - | - |
| 3C | Void | - | - | - | - |
| 3D | The UE associates with the WLAN AP and obtains the local IP address. Note: This could be done by MMI or AT command. The UL signal to WLAN AP shall generate IDC problem. The UE shall detect IDC problem in RRC_IDLE in configuration “C2” with UL signal to WLAN AP. | - | - | - | - |
| 4 | Void | - | - | - | - |
| 4A | The SS re-adjusts the power levels according to row "T2" in Table 8.1.6.1.2.14.3.2-1a. Note: UE shall not detect IDC problem in RRC_IDLE with configuration “T2” | - | - | - | - |
| 5 | Wait 5s for UE to resume logging. | - | - | - | - |
| 6-13 | Steps 1 to 8 of the generic procedure for NR RRC_CONNECTED specified in TS 38.508-1 [4] Table 4.5.4.2-3 are performed. | - | - | - | - |
| 14-20 | Void | - | - | - | - |
| 21 | The SS transmits an UEInformationRequest message to get logMeasReport. | <-- | NR RRC: UEInformationRequest | - | - |
| 22 | Check: Does the UE transmit an UEInformationResponse message including logMeasReport with a LogMeasInfoList consisting of at least three LogMeasInfo entries, one of the LogMeasInfo entries, except for the first one and the last one, is tagged with inDeviceCoexDetected-r17? Note 1: The relativeTimeStamp is increased between the subsequent LogMeasInfoList entries. | --> | NR RRC: UEInformationResponse | 1, 2 | P |

8.1.6.1.2.14.3.3		Specific message contents

Table 8.1.6.1.2.14.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.1.6.1.2.14.3.2-1)

Derivation path: TS 38.508-1 [4], Table 4.6.1-5AA with condition PERIODICAL

Table 8.1.6.1.2.14.3.3-2: UEInformationRequest (steps 21, Table 8.1.6.1.2.14.3.2-1)

Derivation path: TS 38.508-1 [4], Table 4.6.1-32A with condition LOG

Table 8.1.6.1.2.14.3.3-3: UEInformationResponse (step 22, Table 8.1.6.1.2.14.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| logMeasReport-r16 SEQUENCE { |  |  |  |
| absoluteTimeStamp-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceReference-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 SEQUENCE { |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |
| traceId-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |
| traceRecordingSessionRef-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| tce-Id-r16 | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r16 SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-r16 SEQUENCE { | At least 3 entries where at least 3 entry complies to the entries with index ‘x-1’, ‘x’, ‘x+1’ below. SS records the relativeTimeStamp value for each entry |  |  |
| LogMeasInfo-r16[x-1] SEQUENCE { |  | entry x-1 |  |
| locationInfo-r16 | Not checked |  |  |
| relativeTimeStamp-r16 | SS record the value | relativeTimeStamp-r16_x-1 |  |
| servCellIdentity-r16 | Same as NR Cell 14/NR Cell 1 |  |  |
| measResultServCell-16 | Not checked |  |  |
| measResultNeighCells-r16 SEQUENCE { |  |  |  |
| measResultNeighCellListNR | Not present |  |  |
| measResultNeighCellListEUTRA | Not present |  |  |
| } |  |  |  |
| anyCellSelectionDetected-r16 | Not present |  |  |
| inDeviceCoexDetected-r17 | Not present |  |  |
| } |  |  |  |
| LogMeasInfo-r16[x] SEQUENCE { |  | entry x (x can be any but the first and last entry number within logMeasInfoList-r16) |  |
| locationInfo-r16 | Not checked |  |  |
| relativeTimeStamp-r16 | SS record the value | relativeTimeStamp-r16_x |  |
| servCellIdentity-r16 | Same as NR Cell 14/NR Cell 1 |  |  |
| measResultServCell-16 | Not checked |  |  |
| measResultNeighCells-r16 SEQUENCE { |  |  |  |
| measResultNeighCellListNR | Not present |  |  |
| measResultNeighCellListEUTRA | Not present |  |  |
| } |  |  |  |
| anyCellSelectionDetected-r16 | Not present |  |  |
| inDeviceCoexDetected-r17 | true |  |  |
| } |  |  |  |
| LogMeasInfo-r16[x+1] SEQUENCE { |  | entry x+1 |  |
| locationInfo-r16 | Not checked |  |  |
| relativeTimeStamp-r16 | SS record the value | relativeTimeStamp-r16_x+1 |  |
| servCellIdentity-r16 | Same as NR Cell 14/NR Cell 1 |  |  |
| measResultServCell-16 | Not checked |  |  |
| measResultNeighCells-r16 SEQUENCE { |  |  |  |
| measResultNeighCellListNR | Not present |  |  |
| measResultNeighCellListEUTRA | Not present |  |  |
| } |  |  |  |
| anyCellSelectionDetected-r16 | Not present |  |  |
| inDeviceCoexDetected-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| logMeasAvailable-r16 | Not present |  |  |
| logMeasAvailableBT-r16 | Not present |  |  |
| logMeasAvailableWLAN-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

###### 8.1.6.1.2.15	Logged MDT / RRC_IDLE / early measurements

8.1.6.1.2.15.1	Test Purpose (TP)

(1)

with { UE in NR RRC_IDLE state and has already received LoggedMeasurementConfiguration message sending by the network }

ensure that {

when { The earlyMeasIndication-r17 is configured in LoggedMeasurementConfiguration }

then { UE is allowed to log measurements on early measurement frequencies in logged MDT }

}

(2)

with { UE in NR RRC_IDLE state and has already received LoggedMeasurementConfiguration message sending by the network }

ensure that {

when { The earlyMeasIndication-r17 is not configured in LoggedMeasurementConfiguration }

then { UE shall not log measurements on early measurement frequencies in logged MDT }

}

8.1.6.1.2.15.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.331, clause 5.5a.3.2. Unless otherwise stated these are Rel-17 requirements.

[TS 38.331, clause 5.5a.3.2]

While T330 is running and SDT procedure is not ongoing, the UE shall:

1>	if measurement logging is suspended:

2>	if during the last logging interval the IDC problems detected by the UE is resolved, resume measurement logging;

1>	if not suspended, perform the logging in accordance with the following:

…

2>	when performing the logging:

…

3>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include measurements of neighbouring cell that became available during the last logging interval and according to the following:

4>	include measurement results for at most 6 neighbouring cells on the NR serving frequency and for at most 3 cells per NR neighbouring frequency and for the NR neighbouring frequencies in accordance with the following:

5>	if interFreqTargetInfo is included in VarLogMeasConfig:

6>	if earlyMeasIndication is included in VarLogMeasConfig;

7>	include measurement results for NR neighbouring frequencies that are included in both interFreqTargetInfo and either in measIdleCarrierListNR (within the VarMeasIdleConfig) or SIB4;

6>	else:

7>	include measurement results for NR neighbouring frequencies that are included in both interFreqTargetInfo and SIB4;

5>	else:

6>	if earlyMeasIndication is included in VarLogMeasConfig;

7>	include measurement results for NR neighbouring frequencies that are included in either measIdleCarrierListNR (within the VarMeasIdleConfig) or SIB4;

6>	else:

7>	include measurement results for NR neighbouring frequencies that are included in SIB4;

4>	include measurement results for at most 3 neighbours per inter-RAT frequency in accordance with the following:

5>	if earlyMeasIndication is included in VarLogMeasConfig:

6>	include measurement results for inter-RAT neighbouring frequencies that are included in either measIdleCarrierListEUTRA (within the VarMeasIdleConfig) or SIB5;

5>	else:

6>	include measurement results for inter-RAT frequencies that are included in SIB5;

4>	for each neighbour cell included, include the optional fields that are available;

8.1.6.1.2.15.3	Test description

8.1.6.1.2.15.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

UE:

-	None

Preamble:

-	The UE is in state 3N-A as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1.

8.1.6.1.2.15.3.2	Test procedure sequence

Table 8.1.6.1.2.15.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an LoggedMeasurementConfiguration message with earlyMeasIndication set to ‘true’. | <-- | LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits a RRCRelease message with measIdleConfig including measIdleCarrierListNR with ssb-MeasConfig to the UE. | <-- | NR RRC: RRCRelease | - | - |
| 3 | Wait 30 seconds for the UE to perform the logging at regular time intervals. | - | - | - | - |
| 4-11 | Steps 1 to 8 of the generic procedure in TS 38.508-1 [4], Table 4.5.4.2-3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 12 | SS transmits an LoggedMeasurementConfiguration message without earlyMeasIndication. | <-- | LoggedMeasurementConfiguration | - | - |
| 13 | The SS sends an UEInformationRequest message including idleModeMeasurementReq and logMeasReportReq. | <-- | NR RRC: UEInformationRequest | - | - |
| 14 | Check: Does the UE send an UEInformationResponse message with measurement results on early measurement frequencies? | --> | NR RRC: UEInformationResponse | 1 | P |
| 15 | The SS transmits a RRCRelease message with measIdleConfig including measIdleCarrierListNR with ssb-MeasConfig to the UE. | <-- | NR RRC: RRCRelease | - | - |
| 16 | Wait 30 seconds for the UE to perform the logging at regular time intervals. | - | - | - | - |
| 17-24 | Steps 1 to 8 of the generic procedure in TS 38.508-1 [4], Table 4.5.4.2-3 are executed to successfully complete the service request procedure. | - | - | - | - |
| 25 | The SS sends an UEInformationRequest message including idleModeMeasurementReq and logMeasReportReq. | <-- | NR RRC: UEInformationRequest | - | - |
| 26 | Check: Does the UE send an UEInformationResponse message without measurement results on early measurement frequencies? | --> | NR RRC: UEInformationResponse | 2 | P |

8.1.6.1.2.15.3.3	Specific message contents

Table 8.1.6.1.2.15.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.1.6.1.2.15.3.2-1)

| Derivation Path: TS 38.508-1 Table 4.6.1-5AA with condition EarlyMeas |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| loggedMeasurementConfiguration-r16 SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| loggingInterval-r16 | ms61440 | 61.44s |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.2.15.3.3-2: LoggedMeasurementConfiguration (step 12, Table 8.1.6.1.2.15.3.2-1)

| Derivation Path: TS 38.508-1 Table 4.6.1-5AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| loggedMeasurementConfiguration-r16 SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| loggingInterval-r16 | ms61440 | 61.44s |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| earlyMeasIndication-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.2.15.3.3-3: UEInformationRequest (steps 13 and 25, Table 8.1.6.1.2.15.3.2-1)

Derivation path: TS 38.508-1 [4], Table 4.6.1-32A with condition IDLE and LOG

Table 8.1.6.1.2.15.3.3-4: UEInformationResponse (steps 14, Table 8.1.6.1.2.15.3.2-1)

| Derivation Path: TS 38.508-1 [4], table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| measResultIdleNR-r16 SEQUENCE { |  |  |  |
| measResultServingCell-r16 SEQUENCE { |  |  |  |
| rsrp-Result-r16 | Any allowed value |  |  |
| rsrq-Result-r16 | Any allowed value |  |  |
| resultsSSB-Indexes-r16 SEQUENCE { |  |  |  |
| ssb-Index-r16 | Any allowed value |  |  |
| ssb-Results-r16 SEQUENCE { |  |  |  |
| ssb-RSRP-Result-r16 | Any allowed value |  |  |
| ssb-RSRQ-Result-r16 | Any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultsPerCarrierListIdleNR-r16 SEQUENCE (SIZE (1.. maxFreqIdle-r16)) OF MeasResultsPerCarrierIdleNR-r16 { | 1 entry |  |  |
| MeasResultsPerCarrierIdleNR-r16[1] SEQUENCE { |  | entry 1 |  |
| carrierFreq-r16 | ARFCN value corresponding to NR cell 1 |  |  |
| measResultsPerCellListIdleNR-r16 SEQUENCE (SIZE (1..maxCellMeasIdle-r16)) OF MeasResultsPerCellIdleNR-r16 { | 1 entry |  |  |
| MeasResultsPerCellIdleNR-r16[1] SEQUENCE { |  | entry 1 |  |
| physCellId-r16 | PhysCellId corresponding to NR cell 1 |  |  |
| measIdleResultNR-r16 SEQUENCE { |  |  |  |
| rsrp-Result-r16 | Any allowed value |  |  |
| rsrq-Result-r16 | Any allowed value |  |  |
| resultsSSB-Indexes-r16 SEQUENCE { |  |  |  |
| ssb-Index-r16 | Any allowed value |  |  |
| ssb-Results-r16 SEQUENCE { |  |  |  |
| ssb-RSRP-Result-r16 | Any allowed value |  |  |
| ssb-RSRQ-Result-r16 | Any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| logMeasReport-r16 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.2.15.3.3-5: UEInformationResponse (step 26, Table 8.1.6.1.2.15.3.2-1)

| Derivation Path: TS 38.508-1 [4], table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| measResultIdleNR-r16 | Not present |  |  |
| logMeasReport-r16 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

###### 8.1.6.1.2.16	Logged MDT / RRC_IDLE / sig-based logged MDT

8.1.6.1.2.16.1	Test Purpose (TP)

(1)

with { UE in NR RRC_IDLE state with logged measurements available for NR and T330 timer is running }

ensure that {

when { The sigLoggedMeasType is included in VarLogMeasReport }

then { UE includes the sigLogMeasConfigAvailable set to true in the RRCSetupComplete message after receiving an RRCSetup message }

}

8.1.6.1.2.16.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.331, clause 5.3.3.4. Unless otherwise stated these are Rel-17 requirements.

[TS 38.331, clause 5.3.3.4]

The UE shall perform the following actions upon reception of the RRCSetup:

…

1>	set the content of RRCSetupComplete message as follows:

…

2>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable in the RRCSetupComplete message;

3>	if Bluetooth measurement results are included in the logged measurements the UE has available for NR:

4>	include the logMeasAvailableBT in the RRCSetupComplete message;

3>	if WLAN measurement results are included in the logged measurements the UE has available for NR:

4>	include the logMeasAvailableWLAN in the RRCSetupComplete message;

2>	if the sigLoggedMeasType in VarLogMeasReport is included:

3>	if T330 timer is running and the logged measurements configuration is for NR:

4>	set sigLogMeasConfigAvailable to true in the RRCSetupComplete message;

3>	else:

4>	if the UE has logged measurements available for NR:

5>	set sigLogMeasConfigAvailable to false in the RRCSetupComplete message;

8.1.6.1.2.16.3	Test description

8.1.6.1.2.16.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

UE:

-	None

Preamble:

-	The UE is in state 3N-A as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1.

8.1.6.1.2.16.3.2	Test procedure sequence

Table 8.1.6.1.2.16.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an LoggedMeasurementConfiguration message with sigLoggedMeasType set to ‘true’. | <-- | LoggedMeasurementConfiguration | - | - |
| 2 | The SS transmits a RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| 3 | Wait 30 seconds for the UE to perform the logging at regular time intervals. | - | - | - | - |
| 4-6 | Steps 1 to 3 of the generic procedure in TS 38.508-1 [4], Table 4.5.4.2-3 are performed. | - | - | - | - |
| 7 | Check: Does the UE transmit an RRCSetupComplete message including sigLogMeasConfigAvailable IE set it to true when T330 is running? UE initiates the session management procedure by including the SERVICE REQUEST message. | --> | NR RRC: RRCSetupComplete 5GMM: SERVICE REQUEST | 1 | P |
| 8-11 | Steps 5 to 8 of the generic radio bearer establishment procedure in TS 38.508-1 [4] table 4.5.4.2-3. | - | - | - | - |

8.1.6.1.2.16.3.3	Specific message contents

Table 8.1.6.1.2.16.3.3-1: LoggedMeasurementConfiguration (step 1, Table 8.1.6.1.2.16.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-5AA with condition SigLogMDT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| loggedMeasurementConfiguration-r16 SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| loggingInterval-r16 | ms61440 | 61.44s |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.2.16.3.3-2: RRCSetupComplete (step 7, Table 8.1.6.1.2.16.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ue-MeasurementsAvailable-r16 | UE-MeasurementsAvailable-r16 with condition sig_LOG |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.1.6.1.3	Radio Link Failure report

###### 8.1.6.1.3.1	Radio Link Failure / Reporting of Intra-frequency measurements

8.1.6.1.3.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED having performed the intra-frequency measurement and reported that the UE has radio link failure information available }

ensure that {

when { UE receives the UEInformationRequest message containing rlf-ReportReq }

then { UE sends the UEInformationResponse message containing the measurement result for intra-frequency neighbour cell }

}

(2)

with { UE sends the UEInformationResponse message containing the measurement result for intra-frequency neighbour cell }

ensure that {

when { The latest radio measurement results of the serving and neighbouring cells are available }

then { UE includes SSB index and associated measurements in the measResultLastServCell }

}

8.1.6.1.3.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331, clauses 5.3.10.5, 5.3.7.4, 5.3.7.5, 5.3.10.3 and 5.7.10.3. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.3.10.5]

The UE shall determine the content in the VarRLF-Report as follows:

1>	clear the information included in VarRLF-Report, if any;

1>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

1>	set the measResultLastServCell to include the cell level RSRP, RSRQ and the available SINR, of the source PCell(in case HO failure) or PCell (in case RLF) based on the available SSB and CSI-RS measurements collected up to the moment the UE detected failure;

1>	if the SS/PBCH block-based measurement quantities are available:

2>	set the rsIndexResults in measResultLastServCell to include all the available measurement quantities of the source PCell (in case HO failure) or PCell (in case RLF), ordered such that the highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE detected failure;

…

1>	set the ssbRLMConfigBitmap and/or csi-rsRLMConfigBitmap in measResultLastServCell to include the radio link monitoring configuration of the source PCell(in case HO failure) or PCell (in case RLF);

1>	for each of the configured measObjectNR in which measurements are available:

2>	if the SS/PBCH block-based measurement quantities are available:

3>	set the measResultListNR in measResultNeighCells to include all the available measurement quantities of the best measured cells, other than the source PCell(in case HO failure) or PCell (in case RLF), ordered such that the cell with highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the cell with highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the cell with highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE detected failure;

4>	for each neighbour cell included, include the optional fields that are available;

…

1>	set the c-RNTI to the C-RNTI used in the source PCell(in case HO failure) or PCell (in case RLF);

…

1>	else if the failure is detected due to radio link failure as described in 5.3.10.3, set the fields in VarRLF-report as follows:

2>	set the connectionFailureType to rlf;

2>	set the rlf-Cause to the trigger for detecting radio link failure in accordance with clause 5.3.10.4;

2>	set the nrFailedPCellId in failedPCellId to the global cell identity and the tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the PCell where radio link failure is detected;

…

1>	if location information is available, set the content of locationInfo as follows:

2>	if available, set the commonLocationInfo to include the detailed location information;

2>	if available, set the bt-LocationInfo in locationInfo to include the Bluetooth measurement results, in order of decreasing RSSI for Bluetooth beacons;

2>	if available, set the wlan-LocationInfo in locationInfo to include the WLAN measurement results, in order of decreasing RSSI for WLAN APs;

2>	if available, set the sensor-LocationInfo in locationInfo to include the sensor measurement results;

[TS 38.331, clause 5.3.7.4]

The UE shall set the contents of RRCReestablishmentRequest message as follows:

1>	if the procedure was initiated due to radio link failure as specified in 5.3.10.3 or handover failure as specified in 5.3.5.8.3:

2>	set the reestablishmentCellId in the VarRLF-Report to the global cell identity of the selected cell;

[TS 38.331, clause 5.3.7.5]

The UE shall:

…

1>	set the content of RRCReestablishmentComplete message as follows:

2>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable in the RRCReestablishmentComplete message;

…

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report; or

…

3>	include rlf-InfoAvailable in the RRCReestablishmentComplete message;

1>	submit the RRCReestablishmentComplete message to lower layers for transmission;

[TS 38.331, clause 5.3.10.3]

The UE shall:

…

1>	else:

2>	during a DAPS handover: the following only applies for the target PCell;

2>	upon T310 expiry in PCell; or

…

2>	upon consistent uplink LBT failure indication from MCG MAC while T304 is not running:

…

3>	else:

4>	consider radio link failure to be detected for the MCG i.e. RLF;

4>	discard any segments of segmented RRC messages stored according to 5.7.6.3;

NOTE:	Void.

4>	if AS security has not been activated:

5>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other';-

4>	else if AS security has been activated but SRB2 and at least one DRB or, for IAB, SRB2, have not been setup:

5>	store the radio link failure information in the VarRLF-Report as described in subclause 5.3.10.5;

5>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure';

4>	else:

5>	store the radio link failure information in the VarRLF-Report as described in subclause 5.3.10.5;

…

5>	else:

6>	initiate the connection re-establishment procedure as specified in 5.3.7.

The UE may discard the radio link failure information, i.e. release the UE variable VarRLF-Report, 48 hours after the radio link failure is detected.

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if the logMeasReportReq is present and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3>	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3>	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3>	include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3>	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3>	include the logMeasInfoList and set it to include one or more entries from VarLogMeasReport starting from the entries logged first;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

…

1>	if rlf-ReportReq is set to true:

2>	if the UE has radio link failure information or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3>	set timeSinceFailure in VarRLF-Report to the time that elapsed since the last radio link or handover failure in NR;

3>	set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

3>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

…

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1.

8.1.6.1.3.1.3	Test description

8.1.6.1.3.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the serving cell.

-	NR Cell 2 is the intra-frequency neighbor cell.

-	System information combination NR-2 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

None.

Preamble:

-	The UE is in state 3N-A on NR Cell 1 according to TS 38.508-1 [4], clause 4.4A.

8.1.6.1.3.1.3.2	Test procedure sequence

Table 8.1.6.1.3.1.3.2-1/2 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.6.1.3.1.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -94 | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy exit condition for event A3 (M2 < M1). |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -82 | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy entry condition for event A3 (M2 > M1). |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | -82 | Only Cell 2 is available. |

Table 8.1.6.1.3.1.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -91 | -100 | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy exit condition for event A3 (M2 < M1). |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -91 | -82 | The power level values are such that measurement results for Cell 1 (M1) and Cell 2 (M2) satisfy entry condition for event A3 (M2 > M1). |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | -82 | Only Cell 2 is available. |

Table 8.1.6.1.3.1.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message including MeasConfig to setup NR measurement and reporting for intra-frequency event A3 (measId 1). | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | The SS changes NR Cell 2 parameters according to the row "T1" in Table 8.1.6.1.3.1.3.2-1/2. | - | - | - | - |
| 4 | The UE transmit a MeasurementReport message to report event A3 (measId 1) with the measured RSRP value for NR Cell 2. | --> | NR RRC: MeasurementReport | - | - |
| 5 | The SS changes NR Cell 1 parameter according to the row "T2" in Table 8.1.6.1.3.1.3.2-1/2. | - | - | - | - |
| 6 | The UE transmits an RRCReestablishmentRequest message on NR Cell 2. | --> | NR RRC: RRCReestablishmentRequest | - | - |
| 7 | The SS transmits an RRCReestablishment message. | <-- | NR RRC: RRCReestablishment | - | - |
| 8 | Check: Does the UE transmit an RRCReestablishmentComplete message with rlf-InfoAvailable included? | --> | NR RRC: RRCReestablishmentComplete | 1 | P |
| 9 | The SS transmits an RRCReconfiguration message to establish SRB2 and DRB. | <-- | NR RRC: RRCReconfiguration | - | - |
| 10 | The UE transmits an RRCReconfigurationtComplete message. | --> | NR RRC: RRCReconfigurationtComplete | - | - |
| 11 | The SS transmits a UEInformationRequest message. | <-- | NR RRC: UEInformationRequest | - | - |
| 12 | Check: Does the UE transmit a UEInformationResponse message on NR Cell 2? | --> | NR RRC: UEInformationResponse | 1, 2 | P |

8.1.6.1.3.1.3.3	Specific message contents

Table 8.1.6.1.3.1.3.3-1: RRCReconfiguration (step 1, Table 8.1.6.1.3.1.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition NR_MEAS

Table 8.1.6.1.3.1.3.3-2: MeasConfig (Table 8.1.6.1.3.1.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR | Table 8.1.6.1.3.1.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-EventA3 | Table 8.1.6.1.3.1.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.1.3.3-3: MeasObjectNR (Table 8.1.6.1.3.1.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for SSB of NR Cell 1 |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| nrofSS-BlocksToAverage | Not present |  |  |
| } |  |  |  |

Table 8.1.6.1.3.1.3.3-4: ReportConfigNR-EventA3 (Table 8.1.6.1.3.1.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-142 with condition EVENT_A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | 2 | 1 dB(2*0.5 dB) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportAmount | r1 |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | true |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.1.3.3-5: MeasurementReport (step 4, Table 8.1.6.1.3.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-5A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| measurementReport SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServingMOList SEQUENCE (SIZE (1..maxNrofServingCells)) OF MeasResultServMO { | 1 entry | Measurement result for NR Cell 1 |  |
| MeasResultServMO[1] SEQUENCE { |  | entry 1 |  |
| servCellId | ServCellIndex of NR Cell 1 |  |  |
| measResultServingCell SEQUENCE { |  |  |  |
| physCellId | PCI of NR Cell 1 |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListNR SEQUENCE (SIZE (1.. maxCellReport)) OF MeasResultNR { | 1 entry | Measurement result for NR Cell 2 |  |
| MeasResultNR[1] SEQUENCE { |  | entry 1 |  |
| physCellId | PCI of NR Cell 2 |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | Not present |  |  |
| sinr | Not present |  |  |
| } |  |  |  |
| resultsCSI-RS-Cell | Not present |  |  |
| } |  |  |  |
| rsIndexResults | Not present |  |  |
| } |  |  |  |
| cgi-Info | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.1.3.3-6: RRCReestablishmentRequest (step 6, Table 8.1.6.1.3.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReestablishmentRequest ::= SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PCI of NR Cell 1 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS |  |  |
| } |  |  |  |
| reestablishmentCause | otherFailure |  |  |
| } |  |  |  |

Table 8.1.6.1.3.1.3.3-7: RRCReestablishmentComplete (step 8, Table 8.1.6.1.3.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReestablishmentComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ue-MeasurementsAvailable-r16 | UE-MeasurementsAvailable-r16 with condition RLF |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.1.3.3-8: RRCReconfiguration (step 9, Table 8.1.6.1.3.1.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition REEST

Table 8.1.6.1.3.1.3.3-9: UEInformationRequest (step 11, Table 8.1.6.1.3.1.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-32A with condition RLF

Table 8.1.6.1.3.1.3.3-10: UEInformationResponse (step 12, Table 8.1.6.1.3.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| rlf-Report-r16 CHOICE { |  |  |  |
| nr-RLF-Report-r16 SEQUENCE { |  |  |  |
| measResultLastServCell-r16 SEQUENCE { |  |  |  |
| measResult-r16 SEQUENCE { |  |  |  |
| cellResults-r16 SEQUENCE { |  |  |  |
| resultsSSB-Cell-r16 SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| resultsCSI-RS-Cell-r16 | Not checked |  |  |
| } |  |  |  |
| rsIndexResults-r16 SEQUENCE { |  |  |  |
| resultsSSB-Indexes-r16 SEQUENCE { |  |  |  |
| ssb-Index | 1 |  |  |
| ssb-Results SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| ssbRLMConfigBitmap-r16 | 01000000 00000000 00000000 00000000 00000000 00000000 00000000 00000000 |  |  |
| resultsCSI-RS-Indexes-r16 | Not present |  |  |
| csi-rsRLMConfigBitmap-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells-r16 SEQUENCE { |  |  |  |
| measResultListNR-r16 SEQUENCE { |  |  |  |
| ssbFrequency-r16 | ARFCN-ValueNR for SSB of NR Cell 2 |  |  |
| refFreqCSI-RS-r16 | Not present |  |  |
| measResultList-r16 SEQUENCE { |  |  |  |
| physCellId | PCI of NR Cell 2 |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | Not checked |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| rsIndexResults | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| c-RNTI-r16 | the old value of the C-RNTI of the UE used in NR Cell 1 |  |  |
| previousPCellId-r16 | Not present |  |  |
| failedPCellId-r16 CHOICE { |  |  |  |
| nrFailedPCellId-r16 | Any Allowed Value | Either pci-arfcn-r16 or cellGlobalId-r16 corresponding to NR Cell 1 |  |
| eutraFailedPCellId-r16 | Not present |  |  |
| } |  |  |  |
| reconnectCellId-r16 | Not present |  |  |
| timeUntilReconnection-16 | Not present |  |  |
| reestablishmentCellId-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 | PLMN ID of NR Cell 2 |  |  |
| cellIdentity-r16 | CGI of NR Cell 2 |  |  |
| trackingAreaCode-r16 | TAC of NR Cell 2 |  |  |
| } |  |  |  |
| timeConnFailure-r16 | Not present |  |  |
| timeSinceFailure-r16 | Any allowed value |  |  |
| connectionFailureType-r16 | rlf |  |  |
| rlf-Cause-r16 | t310-Expiry |  |  |
| locationInfo-r16 | Not checked |  |  |
| noSuitableCellFound-r16 | Not present |  |  |
| ra-InformationCommon-r16 | Not present |  |  |
| csi-rsRLMConfigBitmap-v1650 | Not present |  |  |
| } |  |  |  |
| eutra-RLF-Report-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

###### 8.1.6.1.3.2	Radio Link Failure / Reporting of Inter-frequency measurements

8.1.6.1.3.2.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED having performed the inter-frequency measurement and reported that the UE has radio link failure information available }

ensure that {

when { UE receives the UEInformationRequest message containing rlf-ReportReq }

then { UE sends the UEInformationResponse message containing the measurement result for inter-frequency neighbour cell }

}

(2)

with { UE sends the UEInformationResponse message containing the measurement result for inter-frequency neighbour cell }

ensure that {

when { The latest radio measurement results of the serving and neighbouring cells are available }

then { UE includs SSB index and associated measurements in the measResultLastServCell }

}

8.1.6.1.3.2.2	Conformance requirements

Same as test case 8.1.6.1.3.1.

8.1.6.1.3.2.3	Test description

8.1.6.1.3.2.3.1	Pre-test conditions

Same as test case 8.1.6.1.3.1 with the following differences:

-	Cells configuration: NR Cell 3 replaces NR Cell 2.

-	System information combination: NR-4 repleaces NR-2.

8.1.6.1.3.2.3.2	Test procedure sequence

Same as test case 8.1.6.1.3.1 with the following differences:

-	Cells configuration: NR Cell 3 replaces NR Cell 2.

8.1.6.1.3.2.3.3	Specific message contents

Same as test case 8.1.6.1.3.1 with the following difference:

-	Cells configuration: NR Cell 3 replaces NR Cell 2.

Table 8.1.6.1.3.2.3.3-1: MeasConfig (Table 8.1.6.1.3.1.3.3-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 2 entries |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-f1 | Table 8.1.6.1.3.2.3.3-2 |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measObjectId | 2 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-f2 | Table 8.1.6.1.3.2.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-EventA3 | Table 8.1.6.1.3.1.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 2 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig |  |  |
| } |  |  |  |

Table 8.1.6.1.3.2.3.3-2: MeasObjectNR-f1 (Table 8.1.6.1.3.2.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for SSB of NR Cell 1 |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| nrofSS-BlocksToAverage | Not present |  |  |
| } |  |  |  |

Table 8.1.6.1.3.2.3.3-3: MeasObjectNR-f2 (Table 8.1.6.1.3.2.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for SSB of NR Cell 3 |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| nrofSS-BlocksToAverage | Not present |  |  |
| } |  |  |  |

Table 8.1.6.1.3.2.3.3-4: ReportConfigNR-EventA3 (Table 8.1.6.1.3.2.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-142 with condition EVENT_A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | 2 | 1 dB (2*0.5 dB) | FR1 |
|  | 2+ Delta(NRf2) - Delta(NRf1) | a3-Offset value set to (2+ Delta(NRf2) - Delta(NRf1)). (Note 1) | FR2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportAmount | r1 |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | true |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| Note 1: Delta(NRf2) and Delta(NRf1) are derived based on calibration procedure defined in the TS 38.508-1 [4], clause 6.1.3.3 where Delta(NRf1) is derived for the carrier of NR PCell and Delta(NRf2) is derived for the carrier of neighbour cell. |  |  |  |

###### 8.1.6.1.3.3	Radio Link Failure / Reporting at RRC connection establishment and reestablishment

8.1.6.1.3.3.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state detecting radio link failure }

ensure that {

when { T301 expires after UE having sent an RRCReestablishmentRequest message, the UE has radio link failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN }

then { UE sends the RRCSetupComplete message with rlf-InfoAvailable included after receiving an RRCSetup message }

}

(2)

with { UE in NR RRC_CONNECTED state detecting radio link failure }

ensure that {

when { UE has radio link failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN }

then { UE sends the RRCReestablishmentComplete message with rlf-InfoAvailable included after receiving an RRCReestablishment message }

}

(3)

with { UE in NR RRC_CONNECTED state with the radio link failure information available and plmn-Identity stored in VarRLF-Report is equal to the RPLMN }

ensure that {

when { UE receives the UEInformationRequest message containing rlf-ReportReq set to true }

then { UE sends the UEInformationResponse message with rlf-Report included }

}

8.1.6.1.3.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331, clauses 5.3.3.4, 5.3.7.5, 5.3.10.3, 5.3.7.4 and 5.7.10.3. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.3.3.4]

The UE shall perform the following actions upon reception of the RRCSetup:

1>	if the RRCSetup is received in response to an RRCReestablishmentRequest; or

…

1>	perform the cell group configuration procedure in accordance with the received masterCellGroup and as specified in 5.3.5.5;

1>	perform the radio bearer configuration procedure in accordance with the received radioBearerConfig and as specified in 5.3.5.6;

1>	if stored, discard the cell reselection priority information provided by the cellReselectionPriorities or inherited from another RAT;

…

1>	set the content of RRCSetupComplete message as follows:

…

2>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable in the RRCSetupComplete message;

…

2>	if the UE has connection establishment failure or connection resume failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

3>	include connEstFailInfoAvailable in the RRCSetupComplete message;

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3>	if reconnectCellId in VarRLF-Report is not set:

4>	set timeUntilReconnection in VarRLF-Report to the time that elapsed since the last radio link or handover failure;

4>	set nrReconnectCellId in reconnectCellId in VarRLF-Report to the global cell identity and the tracking area code of the PCell;

3>	include rlf-InfoAvailable in the RRCSetupComplete message;

…

1>	submit the RRCSetupComplete message to lower layers for transmission, upon which the procedure ends.

[TS 38.331, clause 5.3.7.5]

The UE shall:

…

1>	set the content of RRCReestablishmentComplete message as follows:

2>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable in the RRCReestablishmentComplete message;

…

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report; or

…

3>	include rlf-InfoAvailable in the RRCReestablishmentComplete message;

1>	submit the RRCReestablishmentComplete message to lower layers for transmission;

[TS 38.331, clause 5.3.10.3]

The UE shall:

…

1>	else:

2>	during a DAPS handover: the following only applies for the target PCell;

2>	upon T310 expiry in PCell; or

…

2>	upon consistent uplink LBT failure indication from MCG MAC while T304 is not running:

…

3>	else:

4>	consider radio link failure to be detected for the MCG i.e. RLF;

4>	discard any segments of segmented RRC messages stored according to 5.7.6.3;

NOTE:	Void.

4>	if AS security has not been activated:

5>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other';-

4>	else if AS security has been activated but SRB2 and at least one DRB or, for IAB, SRB2, have not been setup:

5>	store the radio link failure information in the VarRLF-Report as described in subclause 5.3.10.5;

5>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure';

4>	else:

5>	store the radio link failure information in the VarRLF-Report as described in subclause 5.3.10.5;

…

5>	else:

6>	initiate the connection re-establishment procedure as specified in 5.3.7.

The UE may discard the radio link failure information, i.e. release the UE variable VarRLF-Report, 48 hours after the radio link failure is detected.

[TS 38.331, clause 5.3.7.4]

The UE shall set the contents of RRCReestablishmentRequest message as follows:

1>	if the procedure was initiated due to radio link failure as specified in 5.3.10.3 or handover failure as specified in 5.3.5.8.3:

2>	set the reestablishmentCellId in the VarRLF-Report to the global cell identity of the selected cell;

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if the logMeasReportReq is present and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

2>	if VarLogMeasReport includes one or more logged measurement entries, set the contents of the logMeasReport in the UEInformationResponse message as follows:

3>	include the absoluteTimeStamp and set it to the value of absoluteTimeInfo in the VarLogMeasReport;

3>	include the traceReference and set it to the value of traceReference in the VarLogMeasReport;

3>	include the traceRecordingSessionRef and set it to the value of traceRecordingSessionRef in the VarLogMeasReport;

3>	include the tce-Id and set it to the value of tce-Id in the VarLogMeasReport;

3>	include the logMeasInfoList and set it to include one or more entries from VarLogMeasReport starting from the entries logged first;

3>	if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message:

4>	include the logMeasAvailable;

…

1>	if rlf-ReportReq is set to true:

2>	if the UE has radio link failure information or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3>	set timeSinceFailure in VarRLF-Report to the time that elapsed since the last radio link or handover failure in NR;

3>	set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

3>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

…

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1.

8.1.6.1.3.3.3	Test description

8.1.6.1.3.3.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 (TAI-1) is the serving cell.

-	NR Cell 2 and Cell 4 are the inter-frequency neighbor cells.

-	System information combination NR-2 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

NOTE: For NR Cell 2 and Cell 4 TAI is set to TAI-2.

UE:

None.

Preamble:

-	The UE is in state 3N-A on NR Cell 1 according to TS 38.508-1 [4], clause 4.4A.

8.1.6.1.3.3.3.2	Test procedure sequence

Table 8.1.6.1.3.3.3.2-1/2 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.6.1.3.3.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 4 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | "Off" | "Off" | Only Cell 1 is available. |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | -88 | "Off" | Only Cell 2 is available. |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | "Off" | -88 | Only Cell 4 is available. |

Table 8.1.6.1.3.3.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 4 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -82 | "Off" | "Off" | Only Cell 1 is available. |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | -82 | "Off" | Only Cell 2 is available. |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | "Off" | -82 | Only Cell 4 is available. |

Table 8.1.6.1.3.3.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes NR Cell 1 and Cell 2 parameters according to the row "T1" in Table 8.1.6.1.3.3.3.2-1/2. | - | - | - | - |
| 2 | The UE transmits an RRCReestablishmentRequest message on NR Cell 2. | --> | NR RRC: RRCReestablishmentRequest | - | - |
| 3 | The SS does not respond to any RRCReestablishmentRequest message and waits for 1s to ensure that T301 expires and the UE goes to NR RRC_IDLE state on NR Cell 2. | - | - | - | - |
| 4-5 | Steps 1-2 of Table 4.9.5.2.2-1 of the generic procedure in TS 38.508-1 [4] are performed on NR Cell 2. | - | - | - | - |
| 6 | Check: Does the UE transmit an RRCSetupComplete message with rlf-InfoAvailable included and a REGISTRATION REQUEST message indicating "mobility registration updating" is sent to update the registration of the actual tracking area? | --> | NR RRC: RRCSetupComplete 5GMM: REGISTRATION REQUEST | 1 | P |
| 6A | The SS transmits a SecurityModeCommand message. | <-- | NR RRC: SecurityModeCommand | - | - |
| 6B | The UE transmits a SecurityModeComplete message. | --> | NR RRC: SecurityModeComplete | - | - |
| 7-8 | Steps 4-5 of Table 4.9.5.2.2-1 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 8A | The SS transmits an RRCReconfiguration message to establish SRB2 and DRB. | <-- | NR RRC: RRCReconfiguration | - | - |
| 8B | The UE transmits an RRCReconfigurationtComplete message. | --> | NR RRC: RRCReconfigurationtComplete | - | - |
| 9 | The SS changes NR Cell 2 and Cell 4 parameters according to the row "T2" in Table 8.1.6.1.3.3.3.2-1/2. | - | - | - | - |
| 10 | The UE transmits an RRCReestablishmentRequest message on NR Cell 4. | --> | NR RRC: RRCReestablishmentRequest | - | - |
| 11 | The SS transmits an RRCReestablishment message. | <-- | NR RRC: RRCReestablishment | - | - |
| 12 | Check: Does the UE transmit an RRCReestablishmentComplete message with rlf-InfoAvailable included? | --> | NR RRC: RRCReestablishmentComplete | 2 | P |
| 13 | The SS transmits an RRCReconfiguration message to establish SRB2 and DRB. | <-- | NR RRC: RRCReconfiguration | - | - |
| 14 | The UE transmits an RRCReconfigurationtComplete message. | --> | NR RRC: RRCReconfigurationtComplete | - | - |
| 15 | The SS transmits a UEInformationRequest message with rlf-ReportReq set to true. | <-- | NR RRC: UEInformationRequest | - | - |
| 16 | Check: Does the UE transmit a UEInformationResponse message on Cell 4 with rlf-Report included? | --> | NR RRC: UEInformationResponse | 3 | P |

8.1.6.1.3.3.3.3	Specific message contents

Table 8.1.6.1.3.3.3.3-0: RRCSetup (preamble and step 5 in Table 8.1.6.1.3.3.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-21 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetup ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcSetup SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig as per table 8.1.6.1.3.3.3.3-0A | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.3.3.3-0A: CellGroupConfig (preamble)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-19 with condition SRB1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| rlf-TimersAndConstants CHOICE { |  |  |  |
| Setup SEQUENCE { |  |  |  |
| t311 | ms10000 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.3.3.3-1: RRCReestablishmentRequest (steps 2 and 10, Table 8.1.6.1.3.3.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReestablishmentRequest ::= SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PCI of NR Cell 1 |  | Step 2 |
|  | PCI of NR Cell 2 |  | Step 10 |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS |  |  |
| } |  |  |  |
| reestablishmentCause | otherFailure |  |  |
| } |  |  |  |

Table 8.1.6.1.3.3.3.3-2: RRCSetupComplete (step 6, Table 8.1.6.1.3.3.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ue-MeasurementsAvailable-r16 | UE-MeasurementsAvailable-r16 with condition RLF |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.3.3.3-3: RRCReestablishmentComplete (step 12, Table 8.1.6.1.3.3.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReestablishmentComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ue-MeasurementsAvailable-r16 | UE-MeasurementsAvailable-r16 with condition RLF |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.3.3.3-4: UEInformationRequest (step 15, Table 8.1.6.1.3.3.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-32A with condition RLF

Table 8.1.6.1.3.3.3.3-5: UEInformationResponse (step 16, Table 8.1.6.1.3.3.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| rlf-Report-r16 CHOICE { |  |  |  |
| nr-RLF-Report-r16 SEQUENCE { |  |  |  |
| measResultLastServCell-r16 SEQUENCE { |  |  |  |
| measResult-r16 SEQUENCE { |  |  |  |
| cellResults-r16 SEQUENCE { |  |  |  |
| resultsSSB-Cell-r16 SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| resultsCSI-RS-Cell-r16 | Not checked |  |  |
| } |  |  |  |
| rsIndexResults-r16 SEQUENCE { |  |  |  |
| resultsSSB-Indexes-r16 SEQUENCE { |  |  |  |
| ssb-Index | 0 | Cell 2 SSB index |  |
| ssb-Results SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| ssbRLMConfigBitmap-r16 | 10000000 00000000 00000000 00000000 00000000 00000000 00000000 00000000 | Radio LinkMonitoring configuration for Cell 2 |  |
| resultsCSI-RS-Indexes-r16 | Not present |  |  |
| csi-rsRLMConfigBitmap-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells-r16 | Not present |  |  |
| c-RNTI-r16 | the old value of the C-RNTI of the UE used in NR Cell 2 |  |  |
| previousPCellId-r16 | Not present |  |  |
| failedPCellId-r16 CHOICE { |  |  |  |
| nrFailedPCellId-r16 | Any Allowed Value | Either pci-arfcn-r16 or cellGlobalId-r16 corresponding to NR Cell 2 |  |
| eutraFailedPCellId-r16 | Not present |  |  |
| } |  |  |  |
| reconnectCellId-r16 | Not present |  |  |
| timeUntilReconnection-16 | Not present |  |  |
| reestablishmentCellId-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 | PLMN ID of NR Cell 4 |  |  |
| cellIdentity-r16 | CGI of NR Cell 4 |  |  |
| trackingAreaCode-r16 | TAC of NR Cell 4 |  |  |
| } |  |  |  |
| timeConnFailure-r16 | Not present |  |  |
| timeSinceFailure-r16 | Not checked |  |  |
| connectionFailureType-r16 | rlf |  |  |
| rlf-Cause-r16 | t310-Expiry |  |  |
| locationInfo-r16 | Not checked |  |  |
| noSuitableCellFound-r16 | Not present |  |  |
| ra-InformationCommon-r16 | Not present |  |  |
| csi-rsRLMConfigBitmap-v1650 | Not present |  |  |
| } |  |  |  |
| eutra-RLF-Report-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

###### 8.1.6.1.3.4	Radio Link Failure / Reporting at NR handover

8.1.6.1.3.4.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with handover failure information available }

ensure that {

when { UE receives RRCReestablishment message }

then { UE sends the RRCReestablishmentComplete message with rlf-InfoAvailable }

}

(2)

with { UE in RRC_CONNECTED state with the handover failure information available }

ensure that {

when { UE receives the UEInformationRequest message containing rlf-ReportReq }

then { UE sends the UEInformationResponse message with rlf-Report included }

}

8.1.6.1.3.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331, clauses 5.3.7.4, 5.3.10.5, 5.3.5.8.3, 5.3.5.3 and 5.7.10.3. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.3.7.4]

The UE shall set the contents of RRCReestablishmentRequest message as follows:

1>	if the procedure was initiated due to radio link failure as specified in 5.3.10.3 or reconfiguration with sync failure as specified in 5.3.5.8.3:

2>	set the reestablishmentCellId in the VarRLF-Report to the global cell identity of the selected cell;

…

[TS 38.331, clause 5.3.7.4]

The UE shall:

…

…

1>	set the content of RRCReestablishmentComplete message as follows:

…

…

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report of TS 36.331 [10] and if the UE is capable of cross-RAT RLF reporting and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 36.331 [10]:

3>	include rlf-InfoAvailable in the RRCReestablishmentComplete message;

1>	submit the RRCReestablishmentComplete message to lower layers for transmission;

[TS 38.331, clause 5.3.10.5]

The UE shall determine the content in the VarRLF-Report as follows:

1>	clear the information included in VarRLF-Report, if any;

1>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

1>	set the measResultLastServCell to include the cell level RSRP, RSRQ and the available SINR, of the source PCell(in case HO failure) or PCell (in case RLF) based on the available SSB and CSI-RS measurements collected up to the moment the UE detected failure;

1>	if the SS/PBCH block-based measurement quantities are available:

2>	set the rsIndexResults in measResultLastServCell to include all the available measurement quantities of the source PCell (in case HO failure) or PCell (in case RLF), ordered such that the highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE detected failure;

…

1>	set the c-RNTI to the C-RNTI used in the source PCell(in case HO failure) or PCell (in case RLF);

1>	if the failure is detected due to reconfiguration with sync failure as described in 5.3.5.8.3, set the fields in VarRLF-report as follows:

2>	set the connectionFailureType to hof;

2>	set the nrFailedPCellId in failedPCellId to the global cell identity and tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell of the failed handover;

2>	include nrPreviousCell in previousPCellId and set it to the global cell identity and tracking area code of the PCell where the last RRCReconfiguration message including reconfigurationWithSync was received;

2>	set the timeConnFailure to the elapsed time since reception of the last RRCReconfiguration message including the reconfigurationWithSync;

…

1>	if available, set the locationInfo as in 5.3.3.7.

The UE may discard the radio link failure information or handover failure information, i.e. release the UE variable VarRLF-Report, 48 hours after the radio link failure/handover failure is detected.

NOTE 2:	In this clause, the term 'handover failure' has been used to refer to 'reconfiguration with sync failure'.

[TS 38.331, clause 5.3.5.8.3]

The UE shall:

1>	if T304 of the MCG expires:

2>	release dedicated preambles provided in rach-ConfigDedicated if configured;

2>	release dedicated msgA PUSCH resources provided in rach-ConfigDedicated if configured;

…

2>	else:

3>	revert back to the UE configuration used in the source PCell;

3>	store the handover failure information in VarRLF-Report as described in the subclause 5.3.10.5;

3>	initiate the connection re-establishment procedure as specified in subclause 5.3.7.

[TS 38.331, clause 5.3.5.3]

The UE shall perform the following actions upon reception of the RRCReconfiguration, or upon execution of the conditional reconfiguration (CHO or CPC):

…

1>	set the content of the RRCReconfigurationComplete message as follows:

…

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report; or

…

3>	include rlf-InfoAvailable in the RRCReconfigurationComplete message;

…

1>	else (RRCReconfiguration was received via SRB1):

2>	submit the RRCReconfigurationComplete message via SRB1 to lower layers for transmission using the new configuration;

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if rlf-ReportReq is set to true:

2>	if the UE has radio link failure information or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3>	set timeSinceFailure in VarRLF-Report to the time that elapsed since the last radio link failure or handover failure in NR;

3>	set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

3>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

…

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1.

8.1.6.1.3.4.3	Test description

8.1.6.1.3.4.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the serving cell.

-	NR Cell 2 is the Suitable neighbour intra-frequency cell.

-	System information combination NR-2 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

None.

Preamble:

-	The UE is in state 3N-A on NR Cell 1 according to TS 38.508-1 [4], clause 4.4A.

8.1.6.1.3.4.3.2	Test procedure sequence

Table 8.1.6.1.3.4.3.2-1/2 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.6.1.3.4.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -94 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -88 | "Off" | Only Cell 1 is available. |

Table 8.1.6.1.3.4.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -91 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -82 | "Off" | Only Cell 1 is available. |

Table 8.1.6.1.3.4.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform handover to NR Cell 2. | <-- | NR RRC: RRCReconfiguration | - | - |
| - | EXCEPTION: In parallel to the events described in step 2 the steps specified in Table 8.1.6.1.3.4.3.2-4 should take place. | - | - | - | - |
| 2 | The SS changes the power level setting according to the row "T1". | - | - | - | - |
| 3 | The UE transmits an RRCReestablishmentRequest message on NR Cell 1. | --> | NR RRC: RRCReestablishmentRequest | - | - |
| 4 | The SS transmits an RRCReestablishment message to resume SRB1 operation and re-activate security on NR Cell 1. | <-- | NR RRC: RRCReestablishment | - | - |
| 5 | Check: Does the UE transmit an RRCReestablishmentComplete message with rlf-InfoAvailable included using the security key derived from the nextHopChainingCount on NR Cell 1. | --> | NR RRC: RRCReestablishmentComplete | 1 | P |
| 6 | The SS transmits an RRCReconfiguration message to resume existing radio bearers on NR Cell 1. | <-- | NR RRC: RRCReconfiguration | - | - |
| 7 | The UE transmit an RRCReconfigurationtComplete message. | --> | NR RRC: RRCReconfigurationtComplete | - | - |
| 8 | The SS transmits a UEInformationRequest message. | <-- | NR RRC: UEInformationRequest | - | - |
| 9 | Check: Does the UE transmit a UEInformationResponse message on NR Cell 1? | --> | NR RRC: UEInformationResponse | 2 | P |

Table 8.1.6.1.3.4.3.2-4: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: The steps 1 and 2 below are repeated for the duration of T304. | - | - | - | - |
| 1 | The UE attempts to perform the handover using MAC Random Access Preamble on NR Cell 2. | - | - | - | - |
| 2 | The SS does not respond. | - | - | - | - |

8.1.6.1.3.4.3.3	Specific message contents

Table 8.1.6.1.3.4.3.3-1: RRCReconfiguration-HO (step 1, Table 8.1.6.1.3.4.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.8.1-1A with condition RBConfig_KeyChange

Table 8.1.6.1.3.4.3.3-2: RRCReestablishmentRequest (step 3, Table 8.1.6.1.3.4.3.2-3)

| Derivation Path: TS 38.508-1, Table 4.6.1-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReestablishmentRequest ::= SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of NR Cell 1 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the MAC-I value calculated by SS. |  |  |
| } |  |  |  |
| reestablishmentCause | handoverFailure |  |  |
| } |  |  |  |

Table 8.1.6.1.3.4.3.3-3: RRCReestablishment (step 4, Table 8.1.6.1.3.4.3.2-3)

| Derivation Path: TS 38.508-1, Table 4.6.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReestablishment ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReestablishment SEQUENCE { |  |  |  |
| nextHopChainingCount | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.4.3.3-4: RRCReestablishmentComplete (step 5, Table 8.1.6.1.3.4.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReestablishmentComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ue-MeasurementsAvailable-r16 | UE-MeasurementsAvailable-r16 with condition RLF |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.4.3.3-5: RRCReconfiguration (step 6, Table 8.1.6.1.3.4.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition REEST

Table 8.1.6.1.3.4.3.3-6: Void

Table 8.1.6.1.3.4.3.3-7: UEInformationRequest (step 8, Table 8.1.6.1.3.4.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-32A with condition RLF

Table 8.1.6.1.3.4.3.3-8: UEInformationResponse (step 9, Table 8.1.6.1.3.4.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| rlf-Report-r16 CHOICE { |  |  |  |
| nr-RLF-Report-r16 SEQUENCE { |  |  |  |
| measResultLastServCell-r16 SEQUENCE { |  |  |  |
| measResult-r16 SEQUENCE { |  |  |  |
| cellResults-r16 SEQUENCE { |  |  |  |
| resultsSSB-Cell-r16 SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| resultsCSI-RS-Cell-r16 | Not checked |  |  |
| } |  |  |  |
| rsIndexResults-r16 SEQUENCE { |  |  |  |
| resultsSSB-Indexes-r16 SEQUENCE { |  |  |  |
| ssb-Index | 1 |  |  |
| ssb-Results SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| ssbRLMConfigBitmap-r16 | 01000000 00000000 00000000 00000000 00000000 00000000 00000000 00000000 |  |  |
| resultsCSI-RS-Indexes-r16 | Not present |  |  |
| csi-rsRLMConfigBitmap-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells-r16 | Not present |  |  |
| c-RNTI-r16 | the old value of the C-RNTI of the UE used in NR Cell 1 |  |  |
| previousPCellId-r16 CHOICE { |  |  |  |
| nrPreviousCell-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 | PLMN ID of NR Cell 1 |  |  |
| cellIdentity-r16 | CGI of NR Cell 1 |  |  |
| trackingAreaCode-r16 | TAC of NR Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| failedPCellId-r16 CHOICE { |  |  |  |
| nrFailedPCellId-r16 | Any Allowed Value | Either pci-arfcn-r16 or cellGlobalId-r16 corresponding to NR Cell 2 |  |
| eutraFailedPCellId-r16 | Not present |  |  |
| } |  |  |  |
| reconnectCellId-r16 | Not present |  |  |
| timeUntilReconnection-16 | Not present |  |  |
| reestablishmentCellId-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 | PLMN ID of NR Cell 1 |  |  |
| cellIdentity-r16 | CGI of NR Cell 1 |  |  |
| trackingAreaCode-r16 | TAC of NR Cell 1 |  |  |
| } |  |  |  |
| timeConnFailure-r16 | Any allowed value |  |  |
| timeSinceFailure-r16 | Any allowed value |  |  |
| connectionFailureType-r16 | hof |  |  |
| rlf-Cause-r16 | Not checked |  |  |
| locationInfo-r16 | Not checked |  |  |
| noSuitableCellFound-r16 | Not present |  |  |
| ra-InformationCommon-r16 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

###### 8.1.6.1.3.5	Radio Link Failure / Location information

8.1.6.1.3.5.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED having performed the intra-frequency measurement and reported that the UE has radio link failure information with location information available }

ensure that {

when { UE receives the UEInformationRequest message containing rlf-ReportReq }

then { UE sends the UEInformationResponse containing the measurement result for intra-frequency neighbour cell including locationInfo }

}

8.1.6.1.3.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.306 clause 4.2.18; TS 38.331, clauses 5.3.10.3, 5.3.10.5, 5.3.3.7, 5.3.7.4, 5.3.7.5, 5.3.5.9 and 5.7.10.3. Unless otherwise stated these are Rel-16 requirements.

[TS 38.306, clause 4.2.18]

| gnss-Location-r16 Indicates whether the UE is equipped with a GNSS or A-GNSS receiver that may be used to provide detailed location information along with SON or MDT related measurements in RRC_CONNECTED, RRC_IDLE and RRC_INACTIVE. | UE | No | No | No |
| --- | --- | --- | --- | --- |

[TS 38.331, clause 5.3.10.3]

The UE shall:

…

1>	else:

2>	during a DAPS handover: the following only applies for the target PCell;

2>	upon T310 expiry in PCell; or

…

2>	upon consistent uplink LBT failure indication from MCG MAC while T304 is not running:

…

3>	else:

4>	consider radio link failure to be detected for the MCG i.e. RLF;

4>	discard any segments of segmented RRC messages stored according to 5.7.6.3;

NOTE:	Void.

4>	if AS security has not been activated:

5>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other';-

4>	else if AS security has been activated but SRB2 and at least one DRB or, for IAB, SRB2, have not been setup:

5>	store the radio link failure information in the VarRLF-Report as described in subclause 5.3.10.5;

5>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure';

4>	else:

5>	store the radio link failure information in the VarRLF-Report as described in subclause 5.3.10.5;

…

5>	else:

6>	initiate the connection re-establishment procedure as specified in 5.3.7.

The UE may discard the radio link failure information, i.e. release the UE variable VarRLF-Report, 48 hours after the radio link failure is detected.

[TS 38.331, clause 5.3.10.5]

The UE shall determine the content in the VarRLF-Report as follows:

1>	clear the information included in VarRLF-Report, if any;

1>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

1>	set the measResultLastServCell to include the cell level RSRP, RSRQ and the available SINR, of the source PCell(in case HO failure) or PCell (in case RLF) based on the available SSB and CSI-RS measurements collected up to the moment the UE detected failure;

1>	if the SS/PBCH block-based measurement quantities are available:

2>	set the rsIndexResults in measResultLastServCell to include all the available measurement quantities of the source PCell (in case HO failure) or PCell (in case RLF), ordered such that the highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE detected failure;

…

1>	set the ssbRLMConfigBitmap and/or csi-rsRLMConfigBitmap in measResultLastServCell to include the radio link monitoring configuration of the source PCell(in case HO failure) or PCell (in case RLF);

…

1>	set the c-RNTI to the C-RNTI used in the source PCell(in case HO failure) or PCell (in case RLF);

…

1>	else if the failure is detected due to radio link failure as described in 5.3.10.3, set the fields in VarRLF-report as follows:

2>	set the connectionFailureType to rlf;

2>	set the rlf-Cause to the trigger for detecting radio link failure in accordance with clause 5.3.10.4;

2>	set the nrFailedPCellId in failedPCellId to the global cell identity and the tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the PCell where radio link failure is detected;

…

1>	if available, set the locationInfo as in 5.3.3.7.

The UE may discard the radio link failure information or handover failure information, i.e. release the UE variable VarRLF-Report, 48 hours after the radio link failure/handover failure is detected.

NOTE 2:	In this clause, the term 'handover failure' has been used to refer to 'reconfiguration with sync failure'.

[TS 38.331, clause 5.3.3.7]

The UE shall:

1>	if timer T300 expires:

2>	reset MAC, release the MAC configuration and re-establish RLC for all RBs that are established;

2>	if the UE supports RRC Connection Establishment failure with temporary offset and the T300 has expired a consecutive connEstFailCount times on the same cell for which connEstFailureControl is included in SIB1:

3>	for a period as indicated by connEstFailOffsetValidity:

4>	use connEstFailOffset for the parameter Qoffsettemp for the concerned cell when performing cell selection and reselection according to TS 38.304 [20] and TS 36.304 [27];

NOTE 1:	When performing cell selection, if no suitable or acceptable cell can be found, it is up to UE implementation whether to stop using connEstFailOffset for the parameter Qoffsettemp during connEstFailOffsetValidity for the concerned cell.

2>	if the UE has connection establishment failure informaton or connection resume failure information available in VarConnEstFailReport and if the RPLMN is not equal to plmn-identity stored in VarConnEstFailReport; or

2>	if the cell identity of current cell is not equal to the cell identity stored in measResultFailedCell in VarConnEstFailReport:

3>	reset the numberOfConnFail to 0;

2>	clear the content included in VarConnEstFailReport except for the numberOfConnFail, if any;

2>	store the following connection establishment failure information in the VarConnEstFailReport by setting its fields as follows:

…

3>	if available, set the locationInfo as follows:

4>	if available, set the commonLocationInfo to include the detailed location information;

4>	if available, set the bt-LocationInfo to include the Bluetooth measurement results, in order of decreasing RSSI for Bluetooth beacons;

4>	if available, set the wlan-LocationInfo to include the WLAN measurement results, in order of decreasing RSSI for WLAN APs;

4>	if available, set the sensor-LocationInfo to include the sensor measurement results as follows;

5>	if available, include the sensor-MeasurementInformation;

5>	if available, include the sensor-MotionInformation;

[TS 38.331, clause 5.3.7.4]

The UE shall set the contents of RRCReestablishmentRequest message as follows:

1>	if the procedure was initiated due to radio link failure as specified in 5.3.10.3 or handover failure as specified in 5.3.5.8.3:

2>	set the reestablishmentCellId in the VarRLF-Report to the global cell identity of the selected cell;

[TS 38.331, clause 5.3.7.5]

The UE shall:

…

1>	set the content of RRCReestablishmentComplete message as follows:

2>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable in the RRCReestablishmentComplete message;

…

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report; or

…

3>	include rlf-InfoAvailable in the RRCReestablishmentComplete message;

1>	submit the RRCReestablishmentComplete message to lower layers for transmission;

[TS 38.331, clause 5.3.5.9]

The UE shall:

…

1>	if the received otherConfig includes the obtainCommonLocation:

2>	include available detailed location information for any subsequent measurement report or any subsequent RLF report and SCGFailureInformation;

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

1>	if rlf-ReportReq is set to true:

2>	if the UE has radio link failure information or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3>	set timeSinceFailure in VarRLF-Report to the time that elapsed since the last radio link failure or handover failure in NR;

3>	set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

3>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

…

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1.

8.1.6.1.3.5.3	Test description

8.1.6.1.3.5.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the serving cell.

-	NR Cell 2 is the Suitable neighbour intra-frequency cell.

-	System information combination NR-2 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

None.

Preamble:

-	The UE's positioning engine (e.g. standalone GNSS receiver) should be provided with any necessary stimulus to allow it to provide the position. This shall be done by use of the test function Update UE Location Information defined in TS 38.509 [6] or any other suitable method may also be used.

-	The UE is in state 3N-A on NR Cell 1 according to TS 38.508-1 [4], clause 4.4A.

8.1.6.1.3.5.3.2	Test procedure sequence

Same test procedure as specified in 8.1.6.1.3.1.3.2 with the following exception:

-	The specific message content for the RRCReconfiguration message in Table 8.1.6.1.3.1.3.3-1 is replaced by the specific message content in Table 8.1.6.1.3.5.3.3-0.

-	The specific message content for the UEInformationResponse message in Table 8.1.6.1.3.1.3.3-10 is replaced by the specific message content in Table 8.1.6.1.3.5.3.3-1.

8.1.6.1.3.5.3.3	Specific message contents

Table 8.1.6.1.3.5.3.3-0: RRCReconfiguration (step 1, Table 8.1.6.1.3.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition NR_MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| otherConfig-v1610 SEQUENCE { |  |  |  |
| obtainCommonLocation-r16 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.5.3.3-1: UEInformationResponse (step 9, Table 8.1.6.1.3.1.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| rlf-Report-r16 CHOICE { |  |  |  |
| nr-RLF-Report-r16 SEQUENCE { |  |  |  |
| measResultLastServCell-r16 SEQUENCE { |  |  |  |
| measResult-r16 SEQUENCE { |  |  |  |
| cellResults-r16 SEQUENCE { |  |  |  |
| resultsSSB-Cell-r16 SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| resultsCSI-RS-Cell-r16 | Not checked |  |  |
| } |  |  |  |
| rsIndexResults-r16 SEQUENCE { |  |  |  |
| resultsSSB-Indexes-r16 SEQUENCE { |  |  |  |
| ssb-Index | 1 |  |  |
| ssb-Results SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| ssbRLMConfigBitmap-r16 | 01000000 00000000 00000000 00000000 00000000 00000000 00000000 00000000 |  |  |
| resultsCSI-RS-Indexes-r16 | Not present |  |  |
| csi-rsRLMConfigBitmap-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells-r16 SEQUENCE { |  |  |  |
| measResultListNR-r16 SEQUENCE { |  |  |  |
| ssbFrequency-r16 | ARFCN-ValueNR for SSB of NR Cell 2 |  |  |
| refFreqCSI-RS-r16 | Not present |  |  |
| measResultList-r16 SEQUENCE { |  |  |  |
| physCellId | PCI of NR Cell 2 |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | Not present |  |  |
| sinr | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| c-RNTI-r16 | the old value of the C-RNTI of the UE used in NR Cell 1 |  |  |
| previousPCellId-r16 | Not present |  |  |
| failedPCellId-r16 CHOICE { |  |  |  |
| nrFailedPCellId-r16 CHOICE { |  |  |  |
| pci-arfcn-r16 SEQUENCE { |  |  |  |
| physCellId-r16 | PCI of NR Cell 1 |  |  |
| carrierFreq-r16 | NARFCN of NR Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reconnectCellId-r16 | Not present |  |  |
| timeUntilReconnection-16 | Not present |  |  |
| reestablishmentCellId-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 | PLMN ID of NR Cell 2 |  |  |
| cellIdentity-r16 | CGI of NR Cell 2 |  |  |
| trackingAreaCode-r16 | TAC of NR Cell 2 |  |  |
| } |  |  |  |
| timeConnFailure-r16 | Not present |  |  |
| timeSinceFailure-r16 | Any allowed value |  |  |
| connectionFailureType-r16 | rlf |  |  |
| rlf-Cause-r16 | t310-Expiry |  |  |
| locationInfo-r16 SEQUENCE { |  |  |  |
| commonLocationInfo-r16 | Any allowed value |  |  |
| bt-LocationInfo-r16 | Not checked |  |  |
| wlan-LocationInfo-r16 | Not checked |  |  |
| sensor-LocationInfo-r16 | Not checked |  |  |
| } |  |  |  |
| noSuitableCellFound-r16 | Not present |  |  |
| ra-InformationCommon-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

###### 8.1.6.1.3.6	Radio Link Failure / Random access problem

8.1.6.1.3.6.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with the radio link failure information with cause randomAccessProblem available }

ensure that {

when { UE receives the UEInformationRequest message containing rlf-ReportReq }

then { UE sends the UEInformationResponse message with rlf-Report with rlf-cause randomAccesProblem included }

}

8.1.6.1.3.6.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331, clauses 5.3.3.7, 5.7.10.5, 5.3.10.3, 5.3.10.5 and 5.7.10.3. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.3.3.7]

The UE shall:

1>	if timer T300 expires:

…

2>	store the following connection establishment failure information in the VarConnEstFailReport by setting its fields as follows:

3>	set the plmn-Identity to the PLMN selected by upper layers (see TS 24.501 [23]) from the PLMN(s) included in the plmn-IdentityList in SIB1;

…

3>	set perRAInfoList to indicate random access failure information as specified in 5.7.10.5;

…

2>	inform upper layers about the failure to establish the RRC connection, upon which the procedure ends;

The UE may discard the connection establishment failure or connection resume failure information, i.e. release the UE variable VarConnEstFailReport, 48 hours after the last connection establishment failure is detected.

[TS 38.331, clause 5.7.10.5]

The UE shall set the content in ra-InformationCommon as follows:

1>	set the absoluteFrequencyPointA to indicate the absolute frequency of the reference resource block associated to the random-access resources used in the random-access procedure;

1>	set the locationAndBandwidth and subcarrierSpacing associated to the UL BWP of the random-access resources used in the random-access procedure;

1>	set the msg1-FrequencyStart, msg1-FDM and msg1-SubcarrierSpacing associated to the contention based random-access resources if used in the random-access procedure;

1>	set the msg1-FrequencyStartCFRA, msg1-FDMCFRA and msg1-SubcarrierSpacingCFRA associated to the contention free random-access resources if used in the random-access procedure;

1>	set the parameters associated to individual random-access attempt in the chronological order of attempts in the perRAInfoList as follows:

2>	if the random-access resource used is associated to a SS/PBCH block, set the associated random-access parameters for the successive random-access attempts associated to the same SS/PBCH block for one or more random-access attempts as follows:

3>	set the ssb-Index to include the SS/PBCH block index associated to the used random-access resource;

3>	set the numberOfPreamblesSentOnSSB to indicate the number of successive random-access attempts associated to the SS/PBCH block;

3>	for each random-access attempt performed on the random-access resource, include the following parameters in the chronological order of the random-access attempt:

4>	if the random-access attempt is performed on the contention based random-access resource and if raPurpose is not equal to 'requestForOtherSI', include contentionDetected as follows:

5>	if contention resolution was not successful as specified in TS 38.321 [6] for the transmitted preamble:

6>	set the contentionDetected to true;

5>	else:

6>	set the contentionDetected to false;

4>	if the random-access attempt is performed on the contention based random-access resource; or

4>	if the random-access attempt is performed on the contention free random-access resource and if the random-access procedure was initiated due to the PDCCH ordering:

5>	if the SS/PBCH block RSRP of the SS/PBCH block corresponding to the random-access resource used in the random-access attempt is above rsrp-ThresholdSSB:

6>	set the dlRSRPAboveThreshold to true;

5>	else:

6>	set the dlRSRPAboveThreshold to false;

[TS 38.331, clause 5.3.10.3]

The UE shall:

…

1>	else:

…

2>	upon random access problem indication from MCG MAC while neither T300, T301, T304, T311 nor T319 are running; or

[TS 38.331, clause 5.3.10.5]

The UE shall determine the content in the VarRLF-Report as follows:

1>	clear the information included in VarRLF-Report, if any;

1>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

1>	set the measResultLastServCell to include the cell level RSRP, RSRQ and the available SINR, of the source PCell(in case HO failure) or PCell (in case RLF) based on the available SSB and CSI-RS measurements collected up to the moment the UE detected failure;

1>	if the SS/PBCH block-based measurement quantities are available:

2>	set the rsIndexResults in measResultLastServCell to include all the available measurement quantities of the source PCell (in case HO failure) or PCell (in case RLF), ordered such that the highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE detected failure;

…

1>	set the ssbRLMConfigBitmap and/or csi-rsRLMConfigBitmap in measResultLastServCell to include the radio link monitoring configuration of the source PCell(in case HO failure) or PCell (in case RLF);

…

1>	set the c-RNTI to the C-RNTI used in the source PCell(in case HO failure) or PCell (in case RLF);

…

1>	else if the failure is detected due to radio link failure as described in 5.3.10.3, set the fields in VarRLF-report as follows:

2>	set the connectionFailureType to rlf;

2>	set the rlf-Cause to the trigger for detecting radio link failure in accordance with clause 5.3.10.4;

2>	set the nrFailedPCellId in failedPCellId to the global cell identity and the tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the PCell where radio link failure is detected;

…

1>	if connectionFailureType is rlf and the rlf-Cause is set to randomAccessProblem or beamFailureRecoveryFailure; or

1>	if connectionFailureType is hof and if the failed handover is an intra-RAT handover:

2>	set the ra-InformationCommon to include the random-access related information as described in subclause 5.7.10.5;

1>	if available, set the locationInfo as in 5.3.3.7.

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if rlf-ReportReq is set to true:

2>	if the UE has radio link failure information or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3>	set timeSinceFailure in VarRLF-Report to the time that elapsed since the last radio link failure or handover failure in NR;

3>	set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

3>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

…

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1.

8.1.6.1.3.6.3	Test description

8.1.6.1.3.6.3.1	Pre-test conditions

-	NR Cell 1 is the serving cell.

-	System information combination NR-2 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

None.

Preamble:

-	The UE is in state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

8.1.6.1.3.6.3.2	Test procedure sequence

Table 8.1.6.1.3.6.3.2-1: Void

Table 8.1.6.1.3.6.3.2-2: Void

Table 8.1.6.1.3.6.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-11 | Void |  |  |  |  |
| 12 | The SS is configured to ignore the preambles transmitted by the UE on PRACH. | - | - | - | - |
| 13 | The SS sends a PDCCH order with ra-PreambleIndex = '000000'B. | - | - | - | - |
| 14 | The UE transmits Preamble on PRACH preambleTransMax times. | - | - | - | - |
| 14A | The SS is configured to reply the preambles transmitted by the UE on PRACH. |  |  |  |  |
| 15 | The UE transmits an RRCReestablishmentRequest message | --> | NR RRC: RRCReestablishmentRequest | - | - |
| 16 | The SS transmits an RRCReestablishment message. | <-- | NR RRC: RRCReestablishment | - | - |
| 17 | The UE transmita an RRCReestablishmentComplete message. | --> | NR RRC: RRCReestablishmentComplete | - | - |
| 18 | The SS transmits an RRCReconfiguration message to establish SRB2 and DRB. | <-- | NR RRC: RRCReconfiguration | - | - |
| 19 | The UE transmits an RRCReconfigurationtComplete message. | --> | NR RRC: RRCReconfigurationtComplete | - | - |
| 20 | The SS transmits a UEInformationRequest message with rlf-ReportReq set to true. | <-- | NR RRC: UEInformationRequest | - | - |
| 21 | Check: Does the UE transmit a UEInformationResponse message with rlf-Report included? | --> | NR RRC: UEInformationResponse | 2 | P |

8.1.6.1.3.6.3.3	Specific message contents

Table 8.1.6.1.3.6.3.3-1: RRCReestablishmentComplete (step 17, Table 8.1.6.1.3.6.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReestablishmentComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ue-MeasurementsAvailable-r16 | UE-MeasurementsAvailable-r16 with condition RLF |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.6.3.3-2: UEInformationRequest (step 20, Table 8.1.6.1.3.6.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-32A with condition RLF

Table 8.1.6.1.3.6.3.3-3: UEInformationResponse (step 21, Table 8.1.6.1.3.6.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| rlf-Report-r16 CHOICE { |  |  |  |
| nr-RLF-Report-r16 SEQUENCE { |  |  |  |
| measResultLastServCell-r16 SEQUENCE { |  |  |  |
| measResult-r16 SEQUENCE { |  |  |  |
| cellResults-r16 SEQUENCE { |  |  |  |
| resultsSSB-Cell-r16 SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| resultsCSI-RS-Cell-r16 | Not checked |  |  |
| } |  |  |  |
| rsIndexResults-r16 SEQUENCE { |  |  |  |
| resultsSSB-Indexes-r16 SEQUENCE { |  |  |  |
| ssb-Index | 1 |  |  |
| ssb-Results SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| ssbRLMConfigBitmap-r16 | 01000000 00000000 00000000 00000000 00000000 00000000 00000000 00000000 |  |  |
| resultsCSI-RS-Indexes-r16 | Not present |  |  |
| csi-rsRLMConfigBitmap-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells-r16 | Not present |  |  |
| c-RNTI-r16 | the value of the C-RNTI of the UE used in the pre-amble |  |  |
| previousPCellId-r16 CHOICE { | Not present |  |  |
| failedPCellId-r16 CHOICE { |  |  |  |
| nrFailedPCellId-r16 CHOICE { |  |  |  |
| cellGlobalId-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 | PLMN ID of NR Cell 1 |  |  |
| cellIdentity-r16 | CGI of NR Cell 1 |  |  |
| trackingAreaCode-r16 | TAC of NR Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reconnectCellId-r16 | Not present |  |  |
| timeUntilReconnection-16 | Not present |  |  |
| reestablishmentCellId-r16 | Not checked |  |  |
| timeConnFailure-r16 | Not present |  |  |
| timeSinceFailure-r16 | Any allowed value |  |  |
| connectionFailureType-r16 | rlf |  |  |
| rlf-Cause-r16 | randomAccessProblem |  |  |
| locationInfo-r16 | Not checked |  |  |
| noSuitableCellFound-r16 | Not present |  |  |
| ra-InformationCommon-r16 SEQUENCE { |  |  |  |
| absoluteFrequencyPointA-r16 | Any allowed value |  |  |
| locationAndBandwidth-r16 | Any allowed value |  |  |
| subcarrierSpacing-r16 | Any allowed value |  |  |
| msg1-FrequencyStart-r16 | Not checked |  |  |
| msg1-FrequencyStartCFRA-r16 | Not checked |  |  |
| msg1-SubcarrierSpacing-r16 | Not checked |  |  |
| msg1-SubcarrierSpacingCFRA-r16 | Not checked |  |  |
| msg1-FDM-r16 | Not checked |  |  |
| msg1-FDMCFRA-r16 | Not checked |  |  |
| perRAInfoList-r16 SEQUENCE { |  |  |  |
| PerRAInfo-r16[1] CHOICE { |  |  |  |
| perRASSBInfoList-r16 SEQUENCE { |  |  |  |
| PerRASSBInfo-r16[1] SEQUENCE { |  |  |  |
| ssb-Index-r16 | Any allowed value |  |  |
| numberOfPreamblesSentOnSSB-r16 | Any allowed value |  |  |
| perRAAttemptInfoList-r16 SEQUENCE { |  |  |  |
| PerRAAttemptInfo-r16[1] SEQUENCE { |  |  |  |
| contentionDetected-r16 | Any allowed value |  |  |
| dlRSRPAboveThreshold-r16 | True |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

###### 8.1.6.1.3.7	Radio Link Failure / Logging and reporting / Reporting at intra NR handover / PLMN list

8.1.6.1.3.7.1	Test Purpose (TP)

(1)

with { UE having completed the radio bearer establishment and initial security activation procedure }

ensure that {

when { UE performs RRC connection re-establishment procedure and intra-NR handover procedure}

then { UE sends an RRCReconfigurationComplete message with rlf-InfoAvailable }

}

(2)

with { UE receives a UEInformationRequest message with rlf-ReportReq set to true }

ensure that {

when { The RPLMN is included in plmn-IdentityList stored in VarRLF-Report }

then { UE sends a UEInformationResponse message with rlf-Report }

}

8.1.6.1.3.7.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331, clauses 5.3.5.3, 5.3.10.3, 5.3.10.5 and 5.7.10.3. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.3.5.3]

The UE shall perform the following actions upon reception of the RRCReconfiguration, or upon execution of the conditional reconfiguration (CHO or CPC):

…

1>	if the RRCReconfiguration includes the masterCellGroup:

2>	perform the cell group configuration for the received masterCellGroup according to 5.3.5.5;

…

1>	if the RRCReconfiguration message includes the radioBearerConfig:

2>	perform the radio bearer configuration according to 5.3.5.6;

…

1>	set the content of the RRCReconfigurationComplete message as follows:

…

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report; or

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report of TS 36.331 [10] and if the UE is capable of cross-RAT RLF reporting and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 36.331 [10]:

3>	include rlf-InfoAvailable in the RRCReconfigurationComplete message;

[TS 38.331, clause 5.3.10.3]

The UE shall:

…

1>	else:

2>	during a DAPS handover: the following only applies for the target PCell;

2>	upon T310 expiry in PCell; or

2>	upon T312 expiry in PCell; or

2>	upon random access problem indication from MCG MAC while neither T300, T301, T304, T311 nor T319 are running; or

2>	upon indication from MCG RLC that the maximum number of retransmissions has been reached; or

2>	if connected as an IAB-node, upon BH RLF indication received on BAP entity from the MCG; or

2>	upon consistent uplink LBT failure indication from MCG MAC while T304 is not running:

…

3>	else:

4>	consider radio link failure to be detected for the MCG, i.e. MCG RLF;

4>	discard any segments of segmented RRC messages stored according to 5.7.6.3;

NOTE:	Void.

4>	if AS security has not been activated:

5>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other';-

4>	else if AS security has been activated but SRB2 and at least one DRB or, for IAB, SRB2, have not been setup:

5>	store the radio link failure information in the VarRLF-Report as described in subclause 5.3.10.5;

5>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure';

4>	else:

5>	store the radio link failure information in the VarRLF-Report as described in subclause 5.3.10.5;

…

5>	else:

6>	initiate the connection re-establishment procedure as specified in 5.3.7.

[TS 38.331, clause 5.3.10.5]

The UE shall determine the content in the VarRLF-Report as follows:

1>	clear the information included in VarRLF-Report, if any;

1>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

1>	set the measResultLastServCell to include the cell level RSRP, RSRQ and the available SINR, of the source PCell(in case HO failure) or PCell (in case RLF) based on the available SSB and CSI-RS measurements collected up to the moment the UE detected failure;

1>	if the SS/PBCH block-based measurement quantities are available:

2>	set the rsIndexResults in measResultLastServCell to include all the available measurement quantities of the source PCell (in case HO failure) or PCell (in case RLF), ordered such that the highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE detected failure;

…

1>	set the ssbRLMConfigBitmap and/or csi-rsRLMConfigBitmap in measResultLastServCell to include the radio link monitoring configuration of the source PCell(in case HO failure) or PCell (in case RLF);

…

1>	set the c-RNTI to the C-RNTI used in the source PCell(in case HO failure) or PCell (in case RLF);

…

1>	else if the failure is detected due to radio link failure as described in 5.3.10.3, set the fields in VarRLF-report as follows:

2>	set the connectionFailureType to rlf;

2>	set the rlf-Cause to the trigger for detecting radio link failure in accordance with clause 5.3.10.4;

2>	set the nrFailedPCellId in failedPCellId to the global cell identity and the tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the PCell where radio link failure is detected;

…

1>	if available, set the locationInfo as in 5.3.3.7.

The UE may discard the radio link failure information or handover failure information, i.e. release the UE variable VarRLF-Report, 48 hours after the radio link failure/handover failure is detected.

NOTE 2:	In this clause, the term 'handover failure' has been used to refer to 'reconfiguration with sync failure'.

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if rlf-ReportReq is set to true:

2>	if the UE has radio link failure information or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3>	set timeSinceFailure in VarRLF-Report to the time that elapsed since the last radio link failure or handover failure in NR;

3>	set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

3>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

...

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1.

8.1.6.1.3.7.3	Test description

8.1.6.1.3.7.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the serving cell.

-	NR Cell 12 is the Suitable neighbour inter-frequency cell.

-	System information combination NR-4 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

-	The PLMNs are identified in the test by the identifiers in Table 8.1.6.1.3.7.3.1-1.

Table 8.1.6.1.3.7.3.1-1: PLMN identifiers

| Cell | PLMN ID |
| --- | --- |
| 1 | PLMN1 |
| 12 | PLMN2 |

UE:

None.

Preamble:

-	The UE is registered on PLMN1 (NR Cell 1) using the procedure described in TS 38.508-1[4] clause 4.5.2.2-2 except that the REGISTRATION ACCEPT message indicates PLMN2 in the Equivalent PLMN list.

-	The UE is in state 3N-A on NR Cell 1 according to TS 38.508-1 [4], clause 4.4A.

8.1.6.1.3.7.3.2	Test procedure sequence

Table 8.1.6.1.3.7.3.2-1/2 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.6.1.3.7.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 12 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | "Off" |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | "Off" |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -88 | "Off" | Only Cell 1 is available. |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -82 |  |

Table 8.1.6.1.3.7.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 12 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -82 | "Off" |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | "Off" | "Off" |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -82 | "Off" | Only Cell 1 is available. |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -73 |  |

Table 8.1.6.1.3.7.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes NR Cell 1 parameters according to the row "T1" in Table 8.1.6.1.3.7.3.2-1/2. | - | - | - | - |
| 2 | The SS waits for 3s to ensure that the UE detects T310 expiry. | - | - | - | - |
| 3 | The SS changes NR Cell 1 parameters according to the row "T2" in Table 8.1.6.1.3.7.3.2-1/2. | - | - | - | - |
| 4 | The UE transmits an RRCReestablishmentRequest message on NR Cell 1. | --> | NR RRC: RRCReestablishmentRequest | - | - |
| 5 | The SS transmits an RRCReestablishment message. | <-- | NR RRC: RRCReestablishment | - | - |
| 6 | The UE transmits an RRCReestablishmentComplete message. | --> | NR RRC: RRCReestablishmentComplete | - | - |
| 7 | The SS transmits an RRCReconfiguration message to establish SRB2 and DRB. | <-- | NR RRC: RRCReconfiguration | - | - |
| 8 | The UE transmits an RRCReconfigurationtComplete message. | --> | NR RRC: RRCReconfigurationtComplete | - | - |
| 9 | The SS changes NR Cell 12 parameters according to the row "T3" in Table 8.1.6.1.3.7.3.2-1/2. | - |  | - | - |
| 10 | The SS transmits an RRCReconfiguration message on Cell 1 to order the UE to perform inter frequency handover to Cell 12. | <-- | NR RRC: RRCReconfiguration | - | - |
| 11 | Check: Does the UE send an RRCReconfigurationComplete message with rlf-InfoAvailable on Cell 12? | --> | NR RRC: RRCReconfigurationComplete | 1 | P |
| 12 | The UE transmit a REGISTRATION REQUEST message with registration type value set to “mobility registration updating”. | --> | NR RRC: ULInformationTransfer 5GMM: REGISTRATION REQUEST | - | - |
| 13-14 | Steps 4 to 5 of generic test procedure in TS 38.508-1 [4] subclause 4.9.5.2.2-1 are performed on Cell 12. | - | - | - | - |
| 15 | The SS transmits a UEInformationRequest message with rlf-ReportReq set to true. | <-- | NR RRC: UEInformationRequest | - | - |
| 16 | Check: Does the UE transmit a UEInformationResponse message on NR Cell 12? | --> | NR RRC: UEInformationResponse | 2 | P |

8.1.6.1.3.7.3.3	Specific message contents

Table 8.1.6.1.3.7.3.3-0: CellGroupConfig (preamble RRCSetup)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| rlf-TimersAndConstants SEQUENCE { |  |  |  |
| t311 | ms10000 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.7.3.3-1: RRCReestablishmentRequest (step 4, Table 8.1.6.1.3.7.3.2-3)

| Derivation Path: TS 38.508-1, Table 4.6.1-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReestablishmentRequest ::= SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the value of the C-RNTI of the UE |  |  |
| physCellId | PhysicalCellIdentity of NR Cell 1 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the MAC-I value calculated by SS. |  |  |
| } |  |  |  |
| reestablishmentCause | otherFailure |  |  |
| } |  |  |  |

Table 8.1.6.1.3.7.3.3-2: RRCReconfiguration (step 7, Table 8.1.6.1.3.7.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition REEST

Table 8.1.6.1.3.7.3.3-3: RRCReconfiguration-HO (step 10, Table 8.1.6.1.3.7.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.8.1-1A with condition RBConfig_KeyChange

Table 8.1.6.1.3.7.3.3-4: RRCReconfigurationComplete (step 11, Table 8.1.6.1.3.7.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfigurationComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfigurationComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ue-MeasurementsAvailable-r16 | UE-MeasurementsAvailable-r16 with condition RLF |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.7.3.3-5: UEInformationRequest (step 15, Table 8.1.6.1.3.7.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-32A with condition RLF

Table 8.1.6.1.3.7.3.3-6: UEInformationResponse (step 16, Table 8.1.6.1.3.7.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| rlf-Report-r16 CHOICE { |  |  |  |
| nr-RLF-Report-r16 SEQUENCE { |  |  |  |
| measResultLastServCell-r16 SEQUENCE { |  |  |  |
| measResult-r16 SEQUENCE { |  |  |  |
| cellResults-r16 SEQUENCE { |  |  |  |
| resultsSSB-Cell-r16 SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| resultsCSI-RS-Cell-r16 | Not checked |  |  |
| } |  |  |  |
| rsIndexResults-r16 SEQUENCE { |  |  |  |
| resultsSSB-Indexes-r16 SEQUENCE { |  |  |  |
| ssb-Index | 1 |  |  |
| ssb-Results SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| ssbRLMConfigBitmap-r16 | 01000000 00000000 00000000 00000000 00000000 00000000 00000000 00000000 |  |  |
| resultsCSI-RS-Indexes-r16 | Not present |  |  |
| csi-rsRLMConfigBitmap-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells-r16 | Not present |  |  |
| c-RNTI-r16 | the old value of the C-RNTI of the UE used in NR Cell 1 |  |  |
| previousPCellId-r16 | Not present |  |  |
| failedPCellId-r16 CHOICE { |  |  |  |
| nrFailedPCellId-r16 CHOICE { |  |  |  |
| cellGlobalId-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 | the first PLMN entry of plmn-IdentityList (in SIB1) |  |  |
| cellIdentity-r16 | CGI of NR Cell 1 |  |  |
| trackingAreaCode-r16 | TAC of NR Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reconnectCellId-r16 | Not present |  |  |
| timeUntilReconnection-16 | Not present |  |  |
| reestablishmentCellId-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 | the first PLMN entry of plmn-IdentityList (in SIB1) |  |  |
| cellIdentity-r16 | CGI of NR Cell 1 |  |  |
| trackingAreaCode-r16 | TAC of NR Cell 1 |  |  |
| } |  |  |  |
| timeConnFailure-r16 | Not present |  |  |
| timeSinceFailure-r16 | Any allowed value |  |  |
| connectionFailureType-r16 | rlf |  |  |
| rlf-Cause-r16 | t310-Expiry |  |  |
| locationInfo-r16 | Not checked |  |  |
| noSuitableCellFound-r16 | Not present |  |  |
| ra-InformationCommon-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

###### 8.1.6.1.3.8	Radio Link Failure / Logging and reporting / Event A3 / CHO

8.1.6.1.3.8.1	Test Purpose (TP)

(1)

with { UE performed conditional handover to the neighbour cell triggered in conditional configuration execution A3 and UE detected handover failure }

ensure that {

when { UE re-establishes the RRC connection on source cell and receives RRCReestablishment message }

then { UE sends the RRCReestablishmentComplete message with rlf-InfoAvailable }

}

(2)

with { UE performed conditional handover to the neighbour cell triggered in conditional configuration execution A3 and UE detected handover failure }

ensure that {

when { UE receives the UEInformationRequest message containing rlf-ReportReq }

then { UE sends the UEInformationResponse message with rlf-Report for conditional handover failure }

}

8.1.6.1.3.8.2	Conformance requirements

8.1.6.1.3.8.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331, clauses 5.3.7.5, 5.3.10.5, 5.3.5.8.3, 5.3.5.3 and 5.7.10.3. Unless otherwise stated these are Rel-17 requirements.

[TS 38.331, clause 5.3.7.5]

The UE shall:

…

1>	set the content of RRCReestablishmentComplete message as follows:

…

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report of TS 36.331 [10] and if the UE is capable of cross-RAT RLF reporting and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 36.331 [10]:

3>	include rlf-InfoAvailable in the RRCReestablishmentComplete message;

1>	submit the RRCReestablishmentComplete message to lower layers for transmission;

[TS 38.331, clause 5.3.10.5]

The UE shall determine the content in the VarRLF-Report as follows:

1>	clear the information included in VarRLF-Report, if any;

1>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

1>	set the measResultLastServCell to include the cell level RSRP, RSRQ and the available SINR, of the source PCell(in case HO failure) or PCell (in case RLF) based on the available SSB and CSI-RS measurements collected up to the moment the UE detected failure;

1>	if the SS/PBCH block-based measurement quantities are available:

2>	set the rsIndexResults in measResultLastServCell to include all the available measurement quantities of the source PCell (in case HO failure) or PCell (in case RLF), ordered such that the highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE detected failure;

1>	for each of the configured measObjectNR in which measurements are available:

…

2>	for each neighbour cell, if any, included in measResultListNR in measResultNeighCells:

3>	if the UE supports RLF-Report for conditional handover and if the neighbour cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in the MCG VarConditionalReconfig at the moment of the detected failure:

4>	set choConfig in MeasResult2NR to the execution condition for each measId within condTriggerConfig associated to the neighbour cell within the MCG VarConditionalReconfig;

4>	if the first entry of choConfig corresponds to a fulfilled execution condition at the moment of handover failure, or radio link failure; or

4>	if the second entry of choConfig, if available, corresponds to a fulfilled execution condition at the moment of handover failure, or radio link failure:

5>	set firstTriggeredEvent to the execution condition condFirstEvent corresponding to the first entry of choConfig or to the execution condition condSecondEvent corresponding to the second entry of choConfig, whichever execution condition was fulfilled first in time;

5>	set timeBetweenEvents to the elapsed time between the point in time of fullfilling the condition in choConfig that was fulfilled first in time, and the point in time of fullfilling the condition in choConfig that was fulfilled second in time, if both the first execution condition corresponding to the first entry and the second execution condition corresponding to the second entry in the choConfig were fullfilled;

1>	if the failure is detected due to reconfiguration with sync failure as described in 5.3.5.8.3, set the fields in VarRLF-report as follows:

2>	set the connectionFailureType to hof;

2>	if the UE supports RLF-Report for DAPS handover and if any DAPS bearer was configured while T304 was running:

3>	set lastHO-Type to daps;

3>	if radio link failure was detected in the source PCell, according to clause 5.3.10.3:

4>	set timeConnSourceDAPS-Failure to the time between the initiation of the DAPS handover execution and the radio link failure detected in the source PCell while T304 was running;

4>	set the rlf-Cause to the trigger for detecting the source radio link failure in accordance with clause 5.3.10.4;

2>	if the UE supports RLF-Report for conditional handover and if configuration of the conditional handover is available in the MCG VarConditionalReconfig at the moment of the handover failure:

3>	if the UE executed a conditional handover toward target PCell according to the condRRCReconfig of the target PCell:

4>	set timeSinceCHO-Reconfig to the time elapsed between the execution of the last RRCReconfiguration message including reconfigurationWithSync for the target PCell of the failed conditional handover, and the reception in the source PCell of the last conditionalReconfiguration including the condRRCReconfig of the target PCell of the failed conditional handover;

3>	else:

4>	set timeSinceCHO-Reconfig to the time elapsed between the execution of the last RRCReconfiguration message including reconfigurationWithSync for the target PCell of the failed handover, and the reception in the source PCell of the last conditionalReconfiguration including the condRRCReconfig;

3>	set choCandidateCellList to include the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of each of the candidate target cells for conditional handover included in condRRCReconfig within the MCG VarConditionalReconfig at the time of the failed handover, excluding the candidate target cells included in measResulNeighCells;

2>	if the UE supports RLF-Report for conditional handover and if the last executed RRCReconfiguration message including reconfigurationWithSync was concerning a conditional handover:

3>	set lastHO-Type to cho;

2>	set the nrFailedPCellId in failedPCellId to the global cell identity and tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell of the failed handover;

2>	include nrPreviousCell in previousPCellId and set it to the global cell identity and tracking area code of the PCell where the last RRCReconfiguration message including reconfigurationWithSync was received;

2>	set the timeConnFailure to the elapsed time since the execution of the last RRCReconfiguration message including the reconfigurationWithSync;

…

1>	if available, set the locationInfo as in 5.3.3.7.

The UE may discard the radio link failure information or handover failure information, i.e. release the UE variable VarRLF-Report, 48 hours after the radio link failure/handover failure is detected.

NOTE 2:	In this clause, the term 'handover failure' has been used to refer to 'reconfiguration with sync failure'.

[TS 38.331, clause 5.3.5.8.3]

The UE shall:

1>	if T304 of the MCG expires:

2>	release dedicated preambles provided in rach-ConfigDedicated if configured;

2>	release dedicated msgA PUSCH resources provided in rach-ConfigDedicated if configured;

…

2>	else:

3>	revert back to the UE configuration used in the source PCell;

3>	store the handover failure information in VarRLF-Report as described in the subclause 5.3.10.5;

3>	initiate the connection re-establishment procedure as specified in subclause 5.3.7.

[TS 38.331, clause 5.3.5.3]

The UE shall perform the following actions upon reception of the RRCReconfiguration, or upon execution of the conditional reconfiguration (CHO or CPC):

…

1>	set the content of the RRCReconfigurationComplete message as follows:

…

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report; or

…

3>	include rlf-InfoAvailable in the RRCReconfigurationComplete message;

…

1>	else (RRCReconfiguration was received via SRB1):

2>	submit the RRCReconfigurationComplete message via SRB1 to lower layers for transmission using the new configuration;

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if rlf-ReportReq is set to true:

2>	if the UE has radio link failure information or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3>	set timeSinceFailure in VarRLF-Report to the time that elapsed since the last radio link failure or handover failure in NR;

3>	set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

3>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

…

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1.

8.1.6.1.3.8.3	Test description

8.1.6.1.3.8.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the Serving cell and NR Cell 2 is the Suitable neighbour intra-frequency cell.

-	System information combination NR-2 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used for NR Cells.

UE:

-	None.

Preamble:

-	If pc_IP_Ping is set to TRUE then, the UE is in 5GS state 3N-A on NR Cell 1 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

-	Else, the UE is in 5GS state 3N-A and Test Loop Function (On) with UE test loop mode B on NR Cell 1 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

8.1.6.1.3.8.3.2	Test procedure sequence

Table 8.1.6.1.3.8.3.2-1 and Table 8.1.6.1.3.8.3.2-2 illustrates the downlink power levels and other changing parameters to be applied for the NR cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. The configuration "T0" indicates the initial conditions. Subsequent configurations marked "T1" and "T2" are applied at the points indicated in the Main behaviour description in Table 8.1.6.1.3.8.3.2-3.

Table 8.1.6.1.3.8.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -94 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -75 | Power levels are such that entry condition for event A3 is satisfied for NR Cell 2 |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -88 | off |  |

Table 8.1.6.1.3.8.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -91 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -91 | -82 | Power levels are such that entry condition for event A3 is satisfied for NR Cell 2 |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -82 | off |  |

Table 8.1.6.1.3.8.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message including MeasConfig and ConditionalReconfiguration to set NR Cell 2 as the target candidate cell. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message in NR Cell 1. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | SS adjusts the cell-specific reference signal level according to row "T1". | - | - | - | - |
| - | EXCEPTION: The steps 4 and 5 below are repeated for the duration of T304. | - | - | - | - |
| 4 | The UE attempts to perform the intra frequency handover using MAC Random Access Preamble on NR Cell 2. | - | - | - | - |
| 5 | The SS does not respond. | - | - | - | - |
| 6 | SS adjusts the cell-specific reference signal level according to row "T2" right before T304 expire. | - | - | - | - |
| 7 | The UE transmits an RRCReestablishmentRequest message in NR Cell 1. | --> | NR RRC: RRCReestablishmentRequest | - | - |
| 8 | The SS transmits an RRCReestablishment message to resume SRB1 operation and re-activate security in NR Cell 1. | <-- | NR RRC: RRCReestablishment | - | - |
| 9 | Check: Does the UE transmit an RRCReestablishmentComplete message with rlf-InfoAvailable in NR Cell 1? | --> | NR RRC: RRCReestablishmentComplete | 1 | P |
| 10 | The SS transmits an RRCReconfiguration message to resume existing radio bearers on NR Cell 1. | <-- | NR RRC: RRCReconfiguration | - | - |
| 11 | The UE transmits an RRCReconfigurationtComplete message. | --> | NR RRC: RRCReconfigurationtComplete | - | - |
| 12 | The SS transmits a UEInformationRequest message. | <-- | NR RRC: UEInformationRequest | - | - |
| 13 | Check: Does the UE transmit a UEInformationResponse message including rlf-Report for conditional handover failure? | --> | NR RRC: UEInformationResponse | 2 | P |

8.1.6.1.3.8.3.3	Specific message contents

Table 8.1.6.1.3.8.3.3-1: RRCReconfiguration (Step 1, Table 8.1.6.1.3.8.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition NR_MEAS and CHO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| measConfig | MeasConfig | Table 8.1.6.1.3.8.3.3-2 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| conditionalReconfiguration-r16 | ConditionalReconfiguration | Table 8.1.6.1.3.8.3.3-5 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.8.3.3-2: MeasConfig (Table 8.1.6.1.3.8.3.3-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR | Table 8.1.6.1.3.8.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-condEventA3 | Table 8.1.6.1.3.8.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.8.3.3-3: MeasObjectNR (Table 8.1.6.1.3.8.3.3-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR ::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for SSB of NR Cell 1 | The SSB of NR Cell 1 and NR Cell 2 have the same ARFCN value as specified in TS 38.508-1 [4] clause 6.2.3 |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| nrofSS-BlocksToAverage | Not present |  |  |
| } |  |  |  |

Table 8.1.6.1.3.8.3.3-4: ReportConfigNR-condEventA3 (Table 8.1.6.1.3.8.3.3-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-142 with condition CHO AND EVENT_A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| condTriggerConfig-r16 SEQUENCE { |  |  |  |
| condEventId CHOICE { |  |  |  |
| condEventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | 2 | 1 dB(2*0.5 dB) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.8.3.3-5: ConditionalReconfiguration (Table 8.1.6.1.3.8.3.3-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-25D |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConditionalReconfiguration-r16 ::= SEQUENCE { |  |  |  |
| condReconfigToAddModList-r16 | CondReconfigToAddModList-r16 | Table 8.1.6.1.3.8.3.3-6 |  |
| } |  |  |  |

Table 8.1.6.1.3.8.3.3-6: CondReconfigToAddModList-r16 (Table 8.1.6.1.3.8.3.3-5)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-25C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CondReconfigToAddModList-r16 ::= SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigToAddMod-r16 { | 1 entry |  |  |
| CondReconfigToAddMod-r16[1] SEQUENCE { |  | entry 1 |  |
| condReconfigId-r16 | 1 |  |  |
| condExecutionCond-r16 SEQUENCE (SIZE (1..2)) OF MeasId { | 1 entry |  |  |
| MeasId[1] | 1 | entry 1 |  |
| } |  |  |  |
| condRRCReconfig-r16 | RRCReconfiguration-HO | Table 8.1.6.1.3.8.3.3-7 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.8.3.3-7: RRCReconfiguration-HO (Table 8.1.6.1.3.8.3.3-6)

| Derivation Path: TS 38.508-1 [4] Table 4.8.1-1A with condition RBConfig_KeyChange |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| masterCellGroup | CellGroupConfig | Table 8.1.6.1.3.8.3.3-8 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.1.6.1.3.8.3.3-8: CellGroupConfig (Table 8.1.6.1.3.8.3.3-7)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with Condition PCell_change and CFRA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| spCellConfigCommon SEQUENCE { |  |  |  |
| physCellId | Physical Cell Identity of NR Cell 2 |  |  |
| } |  |  |  |
| t304 | ms2000 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.8.3.3-9: RRCReestablishment (step 8, Table 8.1.6.1.3.8.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReestablishment ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReestablishment SEQUENCE { |  |  |  |
| nextHopChainingCount | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.8.3.3-10: RRCReestablishmentComplete (step 9, Table 8.1.6.1.3.8.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReestablishmentComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ue-MeasurementsAvailable-r16 | UE-MeasurementsAvailable-r16 with condition RLF |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.8.3.3-11: RRCReconfiguration (step 10, Table 8.1.6.1.3.8.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition REEST

Table 8.1.6.1.3.8.3.3-12: UEInformationRequest (step 12, Table 8.1.6.1.3.8.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-32A with condition RLF

Table 8.1.6.1.3.8.3.3-13: UEInformationResponse (step 13, Table 8.1.6.1.3.8.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| rlf-Report-r16 CHOICE { |  |  |  |
| nr-RLF-Report-r16 SEQUENCE { |  |  |  |
| measResultLastServCell-r16 SEQUENCE { |  |  |  |
| measResult-r16 SEQUENCE { |  |  |  |
| cellResults-r16 SEQUENCE { |  |  |  |
| resultsSSB-Cell-r16 SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| resultsCSI-RS-Cell-r16 | Not checked |  |  |
| } |  |  |  |
| rsIndexResults-r16 SEQUENCE { |  |  |  |
| resultsSSB-Indexes-r16 SEQUENCE { |  |  |  |
| ssb-Index | 1 |  |  |
| ssb-Results SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| ssbRLMConfigBitmap-r16 | 01000000 00000000 00000000 00000000 00000000 00000000 00000000 00000000 |  |  |
| resultsCSI-RS-Indexes-r16 | Not present |  |  |
| csi-rsRLMConfigBitmap-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells-r16 SEQUENCE { |  |  |  |
| measResultListNR-r16 SEQUENCE (SIZE (1..maxFreq)) OF MeasResult2NR-r16 { | 1 entry |  |  |
| MeasResult2NR-r16[1] SEQUENCE { |  | entry 1 |  |
| ssbFrequency-r16 | Not checked |  |  |
| refFreqCSI-RS-r16 | Not checked |  |  |
| measResultList-r16 SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultNR { | 1 entry |  |  |
| MeasResultNR[1] SEQUENCE { |  | entry 1 |  |
| physCellId | PCI of NR Cell 2 |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | Not present |  |  |
| sinr | Not present |  |  |
| } |  |  |  |
| resultsCSI-RS-Cell | Not present |  |  |
| } |  |  |  |
| rsIndexResults | Not present |  |  |
| } |  |  |  |
| cgi-Info | Not present |  |  |
| choCandidate-r17 | Not present |  |  |
| choConfig-r17 SEQUENCE (SIZE (1..2)) OF CondTriggerConfig-r16 { | 1 entry |  |  |
| CondTriggerConfig-r16[1] SEQUENCE { |  | entry 1 |  |
| condEventId CHOICE { |  |  |  |
| condEventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | 2 | 1 dB (2*0.5 dB) |  |
| } |  |  |  |
| hysteresis | Not checked |  |  |
| timeToTrigger | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| triggeredEvent-r17 SEQUENCE { |  |  |  |
| timeBetweenEvents-r17 | Not present |  |  |
| firstTriggeredEvent | condFirstEvent |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultListEUTRA-r16 | Not present |  |  |
| } |  |  |  |
| c-RNTI-r16 | the old value of the C-RNTI of the UE used in NR Cell 1 |  |  |
| previousPCellId-r16 CHOICE { |  |  |  |
| nrPreviousCell-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 | PLMN ID of NR Cell 1 |  |  |
| cellIdentity-r16 | CGI of NR Cell 1 |  |  |
| trackingAreaCode-r16 | TAC of NR Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| failedPCellId-r16 CHOICE { |  |  |  |
| nrFailedPCellId-r16 | Any Allowed Value | Either pci-arfcn-r16 or cellGlobalId-r16 corresponding to NR Cell 2 |  |
| } |  |  |  |
| reconnectCellId-r16 | Not present |  |  |
| timeUntilReconnection-16 | Not present |  |  |
| reestablishmentCellId-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 | PLMN ID of NR Cell 1 |  |  |
| cellIdentity-r16 | CGI of NR Cell 1 |  |  |
| trackingAreaCode-r16 | TAC of NR Cell 1 |  |  |
| } |  |  |  |
| timeConnFailure-r16 | Any allowed value |  |  |
| timeSinceFailure-r16 | Any allowed value |  |  |
| connectionFailureType-r16 | hof |  |  |
| rlf-Cause-r16 | Not checked |  |  |
| locationInfo-r16 | Not checked |  |  |
| noSuitableCellFound-r16 | Not present |  |  |
| ra-InformationCommon-r16 | Not checked |  |  |
| lastHO-Type-r17 | cho |  |  |
| timeConnSourceDAPS-Failure-r17 | Not checked |  |  |
| timeSinceCHO-Reconfig-r17 | (0..1023) | Less than the value of T304 |  |
| choCellId-r17 | Not checked |  |  |
| choCandidateCellList-r17 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

###### 8.1.6.1.3.9	Radio Link Failure / Logging and reporting / Event A5 / CHO

8.1.6.1.3.9.1	Test Purpose (TP)

(1)

with { UE performed conditional handover to the neighbour cell triggered in conditional configuration execution A5 and UE detected handover failure }

ensure that {

when { UE re-establishes the RRC connection on source cell and receives RRCReestablishment message }

then { UE sends the RRCReestablishmentComplete message with rlf-InfoAvailable }

}

(2)

with { UE performed conditional handover to the neighbour cell triggered in conditional configuration execution A5 and UE detected handover failure }

ensure that {

when { UE receives the UEInformationRequest message containing rlf-ReportReq }

then { UE sends the UEInformationResponse message with rlf-Report for conditional handover failure }

}

8.1.6.1.3.9.2	Conformance requirements

8.1.6.1.3.9.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331, clauses 5.3.7.5, 5.3.10.5, 5.3.5.8.3, 5.3.5.3 and 5.7.10.3. Unless otherwise stated these are Rel-17 requirements.

[TS 38.331, clause 5.3.7.5]

The UE shall:

…

1>	set the content of RRCReestablishmentComplete message as follows:

…

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report of TS 36.331 [10] and if the UE is capable of cross-RAT RLF reporting and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 36.331 [10]:

3>	include rlf-InfoAvailable in the RRCReestablishmentComplete message;

1>	submit the RRCReestablishmentComplete message to lower layers for transmission;

[TS 38.331, clause 5.3.10.5]

The UE shall determine the content in the VarRLF-Report as follows:

1>	clear the information included in VarRLF-Report, if any;

1>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

1>	set the measResultLastServCell to include the cell level RSRP, RSRQ and the available SINR, of the source PCell(in case HO failure) or PCell (in case RLF) based on the available SSB and CSI-RS measurements collected up to the moment the UE detected failure;

1>	if the SS/PBCH block-based measurement quantities are available:

2>	set the rsIndexResults in measResultLastServCell to include all the available measurement quantities of the source PCell (in case HO failure) or PCell (in case RLF), ordered such that the highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE detected failure;

1>	for each of the configured measObjectNR in which measurements are available:

…

2>	for each neighbour cell, if any, included in measResultListNR in measResultNeighCells:

3>	if the UE supports RLF-Report for conditional handover and if the neighbour cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in the MCG VarConditionalReconfig at the moment of the detected failure:

4>	set choConfig in MeasResult2NR to the execution condition for each measId within condTriggerConfig associated to the neighbour cell within the MCG VarConditionalReconfig;

4>	if the first entry of choConfig corresponds to a fulfilled execution condition at the moment of handover failure, or radio link failure; or

4>	if the second entry of choConfig, if available, corresponds to a fulfilled execution condition at the moment of handover failure, or radio link failure:

5>	set firstTriggeredEvent to the execution condition condFirstEvent corresponding to the first entry of choConfig or to the execution condition condSecondEvent corresponding to the second entry of choConfig, whichever execution condition was fulfilled first in time;

5>	set timeBetweenEvents to the elapsed time between the point in time of fullfilling the condition in choConfig that was fulfilled first in time, and the point in time of fullfilling the condition in choConfig that was fulfilled second in time, if both the first execution condition corresponding to the first entry and the second execution condition corresponding to the second entry in the choConfig were fullfilled;

1>	if the failure is detected due to reconfiguration with sync failure as described in 5.3.5.8.3, set the fields in VarRLF-report as follows:

2>	set the connectionFailureType to hof;

2>	if the UE supports RLF-Report for DAPS handover and if any DAPS bearer was configured while T304 was running:

3>	set lastHO-Type to daps;

3>	if radio link failure was detected in the source PCell, according to clause 5.3.10.3:

4>	set timeConnSourceDAPS-Failure to the time between the initiation of the DAPS handover execution and the radio link failure detected in the source PCell while T304 was running;

4>	set the rlf-Cause to the trigger for detecting the source radio link failure in accordance with clause 5.3.10.4;

2>	if the UE supports RLF-Report for conditional handover and if configuration of the conditional handover is available in the MCG VarConditionalReconfig at the moment of the handover failure:

3>	if the UE executed a conditional handover toward target PCell according to the condRRCReconfig of the target PCell:

4>	set timeSinceCHO-Reconfig to the time elapsed between the execution of the last RRCReconfiguration message including reconfigurationWithSync for the target PCell of the failed conditional handover, and the reception in the source PCell of the last conditionalReconfiguration including the condRRCReconfig of the target PCell of the failed conditional handover;

3>	else:

4>	set timeSinceCHO-Reconfig to the time elapsed between the execution of the last RRCReconfiguration message including reconfigurationWithSync for the target PCell of the failed handover, and the reception in the source PCell of the last conditionalReconfiguration including the condRRCReconfig;

3>	set choCandidateCellList to include the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of each of the candidate target cells for conditional handover included in condRRCReconfig within the MCG VarConditionalReconfig at the time of the failed handover, excluding the candidate target cells included in measResulNeighCells;

2>	if the UE supports RLF-Report for conditional handover and if the last executed RRCReconfiguration message including reconfigurationWithSync was concerning a conditional handover:

3>	set lastHO-Type to cho;

2>	set the nrFailedPCellId in failedPCellId to the global cell identity and tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell of the failed handover;

2>	include nrPreviousCell in previousPCellId and set it to the global cell identity and tracking area code of the PCell where the last RRCReconfiguration message including reconfigurationWithSync was received;

2>	set the timeConnFailure to the elapsed time since the execution of the last RRCReconfiguration message including the reconfigurationWithSync;

…

1>	if available, set the locationInfo as in 5.3.3.7.

The UE may discard the radio link failure information or handover failure information, i.e. release the UE variable VarRLF-Report, 48 hours after the radio link failure/handover failure is detected.

NOTE 2:	In this clause, the term 'handover failure' has been used to refer to 'reconfiguration with sync failure'.

[TS 38.331, clause 5.3.5.8.3]

The UE shall:

1>	if T304 of the MCG expires:

2>	release dedicated preambles provided in rach-ConfigDedicated if configured;

2>	release dedicated msgA PUSCH resources provided in rach-ConfigDedicated if configured;

…

2>	else:

3>	revert back to the UE configuration used in the source PCell;

3>	store the handover failure information in VarRLF-Report as described in the subclause 5.3.10.5;

3>	initiate the connection re-establishment procedure as specified in subclause 5.3.7.

[TS 38.331, clause 5.3.5.3]

The UE shall perform the following actions upon reception of the RRCReconfiguration, or upon execution of the conditional reconfiguration (CHO or CPC):

…

1>	set the content of the RRCReconfigurationComplete message as follows:

…

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report; or

…

3>	include rlf-InfoAvailable in the RRCReconfigurationComplete message;

…

1>	else (RRCReconfiguration was received via SRB1):

2>	submit the RRCReconfigurationComplete message via SRB1 to lower layers for transmission using the new configuration;

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if rlf-ReportReq is set to true:

2>	if the UE has radio link failure information or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3>	set timeSinceFailure in VarRLF-Report to the time that elapsed since the last radio link failure or handover failure in NR;

3>	set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

3>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

…

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1.

8.1.6.1.3.9.3	Test description

8.1.6.1.3.9.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the Serving cell and NR Cell 2 is the Suitable neighbour intra-frequency cell.

-	System information combination NR-2 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used for NR Cells.

UE:

-	None.

Preamble:

-	If pc_IP_Ping is set to TRUE then, the UE is in 5GS state 3N-A on NR Cell 1 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

-	Else, the UE is in 5GS state 3N-A and Test Loop Function (On) with UE test loop mode B on NR Cell 1 according to TS 38.508-1[4], clause 4.4A.2 Table 4.4A.2-3.

8.1.6.1.3.9.3.2	Test procedure sequence

Table 8.1.6.1.3.9.3.2-1 and Table 8.1.6.1.3.9.3.2-2 illustrate the downlink power levels and other changing parameters to be applied for the NR cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. The configuration "T0" indicates the initial conditions. Subsequent configurations marked "T1" and "T2" are applied at the points indicated in the Main behaviour description in Table 8.1.6.1.3.9.3.2-3.

Table 8.1.6.1.3.9.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -94 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -91 | -75 | Power levels are such that entry condition for event A5 is satisfied for NR Cell 2 |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -88 | off |  |

Table 8.1.6.1.3.9.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -91 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -91 | -82 | Power levels are such that entry condition for event A5 is satisfied for NR Cell 2 |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -82 | off |  |

Table 8.1.6.1.3.9.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message including MeasConfig and ConditionalReconfiguration to set NR Cell 2 as the target candidate cell. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message in NR Cell 1. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | SS adjusts the cell-specific reference signal level according to row "T1". | - | - | - | - |
| - | EXCEPTION: The steps 4 and 5 below are repeated for the duration of T304. | - | - | - | - |
| 4 | The UE attempts to perform the intra frequency handover using MAC Random Access Preamble on NR Cell 2. | - | - | - | - |
| 5 | The SS does not respond. | - | - | - | - |
| 6 | SS adjusts the cell-specific reference signal level according to row "T2" right before T304 expire. | - | - | - | - |
| 7 | The UE transmits an RRCReestablishmentRequest message in NR Cell 1. | --> | NR RRC: RRCReestablishmentRequest | - | - |
| 8 | The SS transmits an RRCReestablishment message to resume SRB1 operation and re-activate security in NR Cell 1. | <-- | NR RRC: RRCReestablishment | - | - |
| 9 | Check: Does the UE transmit an RRCReestablishmentComplete message with rlf-InfoAvailable in NR Cell 1? | --> | NR RRC: RRCReestablishmentComplete | 1 | P |
| 10 | The SS transmits an RRCReconfiguration message to resume existing radio bearers on NR Cell 1. | <-- | NR RRC: RRCReconfiguration | - | - |
| 11 | The UE transmits an RRCReconfigurationtComplete message. | --> | NR RRC: RRCReconfigurationtComplete | - | - |
| 12 | The SS transmits a UEInformationRequest message. | <-- | NR RRC: UEInformationRequest | - | - |
| 13 | Check: Does the UE transmit a UEInformationResponse message including rlf-Report for conditional handover failure? | --> | NR RRC: UEInformationResponse | 2 | P |

8.1.6.1.3.9.3.3	Specific message contents

Table 8.1.6.1.3.9.3.3-1: RRCReconfiguration (Step 1, Table 8.1.6.1.3.9.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition NR_MEAS and CHO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| measConfig | MeasConfig | Table 8.1.6.1.3.9.3.3-2 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| conditionalReconfiguration-r16 | ConditionalReconfiguration | Table 8.1.6.1.3.9.3.3-5 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.9.3.3-2: MeasConfig (Table 8.1.6.1.3.9.3.3-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR | Table 8.1.6.1.3.9.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-condEventA5 | Table 8.1.6.1.3.9.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.9.3.3-3: MeasObjectNR (Table 8.1.6.1.3.9.3.3-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR ::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for SSB of NR Cell 1 | The SSB of NR Cell 1 and NR Cell 2 have the same ARFCN value as specified in TS 38.508-1 [4] clause 6.2.3 |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| nrofSS-BlocksToAverage | Not present |  |  |
| } |  |  |  |

Table 8.1.6.1.3.9.3.3-4: ReportConfigNR-condEventA5 (Table 8.1.6.1.3.9.3.3-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-142 with condition CHO AND EVENT_ A5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| condTriggerConfig-r16 SEQUENCE { |  |  |  |
| condEventId CHOICE { |  |  |  |
| condEventA5 SEQUENCE { |  |  |  |
| a5-Threshold1 CHOICE { |  |  |  |
| rsrp | 76 | -80dBm | FR1 |
|  | 66 + Delta(NRfs) | Threshold set to -91dBm | FR2 |
| } |  |  |  |
| a5-Threshold2 CHOICE { |  |  |  |
| rsrp | 70 | -86dBm | FR1 |
|  | 66 + Delta(NRfs) | Threshold set to -91dBm | FR2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.9.3.3-5: ConditionalReconfiguration (Table 8.1.6.1.3.9.3.3-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-25D |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConditionalReconfiguration-r16 ::= SEQUENCE { |  |  |  |
| condReconfigToAddModList-r16 | CondReconfigToAddModList-r16 | Table 8.1.6.1.3.9.3.3-6 |  |
| } |  |  |  |

Table 8.1.6.1.3.9.3.3-6: CondReconfigToAddModList-r16 (Table 8.1.6.1.3.9.3.3-5)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-25C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CondReconfigToAddModList-r16 ::= SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigToAddMod-r16 { | 1 entry |  |  |
| CondReconfigToAddMod-r16[1] SEQUENCE { |  | entry 1 |  |
| condReconfigId-r16 | 1 |  |  |
| condExecutionCond-r16 SEQUENCE (SIZE (1..2)) OF MeasId { | 1 entry |  |  |
| MeasId[1] | 1 | entry 1 |  |
| } |  |  |  |
| condRRCReconfig-r16 | RRCReconfiguration-HO | Table 8.1.6.1.3.9.3.3-7 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.9.3.3-7: RRCReconfiguration-HO (Table 8.1.6.1.3.9.3.3-6)

| Derivation Path: TS 38.508-1 [4] Table 4.8.1-1A with condition RBConfig_KeyChange |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| masterCellGroup | CellGroupConfig | Table 8.1.6.1.3.9.3.3-8 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.1.6.1.3.9.3.3-8: CellGroupConfig (Table 8.1.6.1.3.9.3.3-7)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with Condition PCell_change and CFRA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| spCellConfigCommon SEQUENCE { |  |  |  |
| physCellId | Physical Cell Identity of NR Cell 2 |  |  |
| } |  |  |  |
| t304 | ms2000 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.9.3.3-9: RRCReestablishment (step 8, Table 8.1.6.1.3.9.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReestablishment ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReestablishment SEQUENCE { |  |  |  |
| nextHopChainingCount | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.9.3.3-10: RRCReestablishmentComplete (step 9, Table 8.1.6.1.3.9.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReestablishmentComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ue-MeasurementsAvailable-r16 | UE-MeasurementsAvailable-r16 with condition RLF |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.9.3.3-11: RRCReconfiguration (step 10, Table 8.1.6.1.3.9.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition REEST

Table 8.1.6.1.3.9.3.3-12: UEInformationRequest (step 12, Table 8.1.6.1.3.9.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-32A with condition RLF

Table 8.1.6.1.3.9.3.3-13: UEInformationResponse (step 13, Table 8.1.6.1.3.9.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| rlf-Report-r16 CHOICE { |  |  |  |
| nr-RLF-Report-r16 SEQUENCE { |  |  |  |
| measResultLastServCell-r16 SEQUENCE { |  |  |  |
| measResult-r16 SEQUENCE { |  |  |  |
| cellResults-r16 SEQUENCE { |  |  |  |
| resultsSSB-Cell-r16 SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| resultsCSI-RS-Cell-r16 | Not checked |  |  |
| } |  |  |  |
| rsIndexResults-r16 SEQUENCE { |  |  |  |
| resultsSSB-Indexes-r16 SEQUENCE { |  |  |  |
| ssb-Index | 1 |  |  |
| ssb-Results SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| ssbRLMConfigBitmap-r16 | 01000000 00000000 00000000 00000000 00000000 00000000 00000000 00000000 |  |  |
| resultsCSI-RS-Indexes-r16 | Not present |  |  |
| csi-rsRLMConfigBitmap-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells-r16 SEQUENCE { |  |  |  |
| measResultListNR-r16 SEQUENCE SIZE (1..maxFreq)) OF MeasResult2NR-r16 { | 1 entry |  |  |
| MeasResult2NR-r16[1] SEQUENCE { |  | entry 1 |  |
| ssbFrequency-r16 | Not checked |  |  |
| refFreqCSI-RS-r16 | Not checked |  |  |
| measResultList-r16 SEQUENCE SIZE (1..maxCellReport)) OF MeasResultNR { | 1 entry |  |  |
| MeasResultNR[1] SEQUENCE { |  | entry 1 |  |
| physCellId | PCI of NR Cell 2 |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | Not present |  |  |
| sinr | Not present |  |  |
| } |  |  |  |
| resultsCSI-RS-Cell | Not present |  |  |
| } |  |  |  |
| rsIndexResults | Not present |  |  |
| } |  |  |  |
| cgi-Info | Not present |  |  |
| choCandidate-r17 | Not present |  |  |
| choConfig-r17 SEQUENCE (SIZE (1..2)) OF CondTriggerConfig-r16 { | 1 entry |  |  |
| CondTriggerConfig-r16[1] SEQUENCE { |  | entry 1 |  |
| condEventId CHOICE { |  |  |  |
| condEventA5 SEQUENCE { |  |  |  |
| a5-Threshold1 CHOICE { |  |  |  |
| rsrp | 76 | -80dBm | FR1 |
|  | 66 + Delta(NRfs) | Threshold set to -91dBm | FR2 |
| } |  |  |  |
| a5-Threshold2 CHOICE { |  |  |  |
| rsrp | 70 | -86dBm | FR1 |
|  | 66 + Delta(NRfs) | Threshold set to -91dBm | FR2 |
| } |  |  |  |
| hysteresis | Not checked |  |  |
| timeToTrigger | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| triggeredEvent-r17 SEQUENCE { |  |  |  |
| timeBetweenEvents-r17 | Not present |  |  |
| firstTriggeredEvent | condFirstEvent |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultListEUTRA-r16 | Not present |  |  |
| } |  |  |  |
| c-RNTI-r16 | the old value of the C-RNTI of the UE used in NR Cell 1 |  |  |
| previousPCellId-r16 CHOICE { |  |  |  |
| nrPreviousCell-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 | PLMN ID of NR Cell 1 |  |  |
| cellIdentity-r16 | CGI of NR Cell 1 |  |  |
| trackingAreaCode-r16 | TAC of NR Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| failedPCellId-r16 CHOICE { |  |  |  |
| nrFailedPCellId-r16 | Any Allowed Value | Either pci-arfcn-r16 or cellGlobalId-r16 corresponding to NR Cell 2 |  |
| } |  |  |  |
| reconnectCellId-r16 | Not present |  |  |
| timeUntilReconnection-16 | Not present |  |  |
| reestablishmentCellId-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 | PLMN ID of NR Cell 1 |  |  |
| cellIdentity-r16 | CGI of NR Cell 1 |  |  |
| trackingAreaCode-r16 | TAC of NR Cell 1 |  |  |
| } |  |  |  |
| timeConnFailure-r16 | Any allowed value |  |  |
| timeSinceFailure-r16 | Any allowed value |  |  |
| connectionFailureType-r16 | hof |  |  |
| rlf-Cause-r16 | Not checked |  |  |
| locationInfo-r16 | Not checked |  |  |
| noSuitableCellFound-r16 | Not present |  |  |
| ra-InformationCommon-r16 | Not checked |  |  |
| lastHO-Type-r17 | cho |  |  |
| timeConnSourceDAPS-Failure-r17 | Not checked |  |  |
| timeSinceCHO-Reconfig-r17 | (0..1023) | Less than the value of T304 |  |
| choCellId-r17 | Not checked |  |  |
| choCandidateCellList-r17 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

###### 8.1.6.1.3.10	Radio Link Failure / Logging and reporting / DAPS HO

8.1.6.1.3.10.1	Test Purpose (TP)

(1)

with { UE having detected DAPS handover failure and radio link failure in source PCell while T304 was running }

ensure that {

when { UE receives RRCReestablishment message }

then { UE sends the RRCReestablishmentComplete message with rlf-InfoAvailable }

}

(2)

with { UE having detected DAPS handover failure and radio link failure in source PCell while T304 was running }

ensure that {

when { UE receives the UEInformationRequest message containing rlf-ReportReq }

then { UE sends the UEInformationResponse message with rlf-Report for DAPS handover failure }

}

8.1.6.1.3.10.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331, clauses 5.3.7.5, 5.3.10.3, 5.3.10.5, 5.3.5.8.3, 5.3.5.3 and 5.7.10.3. Unless otherwise stated these are Rel-17 requirements.

[TS 38.331, clause 5.3.7.5]

The UE shall:

…

1>	set the content of RRCReestablishmentComplete message as follows:

…

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report of TS 36.331 [10] and if the UE is capable of cross-RAT RLF reporting and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 36.331 [10]:

3>	include rlf-InfoAvailable in the RRCReestablishmentComplete message;

1>	submit the RRCReestablishmentComplete message to lower layers for transmission;

[TS 38.331, clause 5.3.10.3]

The UE shall:

1>	if any DAPS bearer is configured and T304 is running:

2>	upon T310 expiry in source SpCell; or

2>	upon random access problem indication from source MCG MAC; or

2>	upon indication from source MCG RLC that the maximum number of retransmissions has been reached; or

2>	upon consistent uplink LBT failure indication from source MCG MAC:

3>	consider radio link failure to be detected for the source MCG i.e. source RLF;

3>	suspend the transmission and reception of all DRBs and multicast MRBs in the source MCG;

3>	reset MAC for the source MCG;

3>	release the source connection.

[TS 38.331, clause 5.3.10.5]

The UE shall determine the content in the VarRLF-Report as follows:

1>	clear the information included in VarRLF-Report, if any;

1>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

1>	set the measResultLastServCell to include the cell level RSRP, RSRQ and the available SINR, of the source PCell(in case HO failure) or PCell (in case RLF) based on the available SSB and CSI-RS measurements collected up to the moment the UE detected failure;

1>	if the SS/PBCH block-based measurement quantities are available:

2>	set the rsIndexResults in measResultLastServCell to include all the available measurement quantities of the source PCell (in case HO failure) or PCell (in case RLF), ordered such that the highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE detected failure;

…

1>	if the failure is detected due to reconfiguration with sync failure as described in 5.3.5.8.3, set the fields in VarRLF-report as follows:

2>	set the connectionFailureType to hof;

2>	if any DAPS bearer was configured while T304 was running:

3>	set lastHO-Type to daps;

3>	if radio link failure was detected in the source PCell, according to clause 5.3.10.3:

4>	set timeConnSourceDAPS-Failure to the time between the initiation of the DAPS handover execution and the radio link failure detected in the source PCell while T304 was running;

4>	set the rlf-Cause to the trigger for detecting the source radio link failure in accordance with clause 5.3.10.4;

2>	set the nrFailedPCellId in failedPCellId to the global cell identity and tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell of the failed handover;

2>	include nrPreviousCell in previousPCellId and set it to the global cell identity and tracking area code of the PCell where the last RRCReconfiguration message including reconfigurationWithSync was received;

2>	set the timeConnFailure to the elapsed time since reception of the last RRCReconfiguration message including the reconfigurationWithSync;

…

1>	if available, set the locationInfo as in 5.3.3.7.

The UE may discard the radio link failure information or handover failure information, i.e. release the UE variable VarRLF-Report, 48 hours after the radio link failure/handover failure is detected.

NOTE 2:	In this clause, the term 'handover failure' has been used to refer to 'reconfiguration with sync failure'.

[TS 38.331, clause 5.3.5.8.3]

The UE shall:

1>	if T304 of the MCG expires:

2>	release dedicated preambles provided in rach-ConfigDedicated if configured;

2>	release dedicated msgA PUSCH resources provided in rach-ConfigDedicated if configured;

…

2>	else:

3>	revert back to the UE configuration used in the source PCell;

3>	store the handover failure information in VarRLF-Report as described in the subclause 5.3.10.5;

3>	initiate the connection re-establishment procedure as specified in subclause 5.3.7.

[TS 38.331, clause 5.3.5.3]

The UE shall perform the following actions upon reception of the RRCReconfiguration, or upon execution of the conditional reconfiguration (CHO or CPC):

…

1>	set the content of the RRCReconfigurationComplete message as follows:

…

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report; or

…

3>	include rlf-InfoAvailable in the RRCReconfigurationComplete message;

…

1>	else (RRCReconfiguration was received via SRB1):

2>	submit the RRCReconfigurationComplete message via SRB1 to lower layers for transmission using the new configuration;

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if rlf-ReportReq is set to true:

2>	if the UE has radio link failure information or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3>	set timeSinceFailure in VarRLF-Report to the time that elapsed since the last radio link failure or handover failure in NR;

3>	set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

3>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

…

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1.

8.1.6.1.3.10.3	Test description

8.1.6.1.3.10.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the Serving cell and NR Cell 2 is the Suitable neighbour intra-frequency cell.

-	System information combination NR-2 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used for NR Cells.

UE:

-	None.

Preamble:

-	If pc_IP_Ping is set to TRUE then, the UE is in 5GS state 3N-A on NR Cell 1 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

-	Else, the UE is in 5GS state 3N-A and Test Loop Function (On) with UE test loop mode B on NR Cell 1 according to TS 38.508-1[4], clause 4.4A.2 Table 4.4A.2-3.

8.1.6.1.3.10.3.2	Test procedure sequence

Table 8.1.6.1.3.10.3.2-1 and Table 8.1.6.1.3.10.3.2-2 illustrates the downlink power levels and other changing parameters to be applied for the NR cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. The configuration "T0" indicates the initial conditions. Subsequent configurations marked "T1", "T2" are applied at the points indicated in the Main behaviour description in Table 8.1.6.1.3.10.3.2-3.

Table 8.1.6.1.3.10.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -94 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | off | off |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -88 | off |  |

Table 8.1.6.1.3.10.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -91 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | off | off |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -82 | off |  |

Table 8.1.6.1.3.10.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message containing reconfigurationWithSync to order UE to perform DAPS handover to NR Cell 2. DRB#n of the first PDU session is configured as DAPS bearer. | <-- | NR RRC: RRCReconfiguration | - | - |
| - | EXCEPTION: In parallel with step 2, parallel behaviour defined in table 8.1.6.1.3.10.3.2-4 is executed repeatedly. | - | - | - | - |
| 2 | The SS changes the power level setting according to the row "T1". | - | - | - | - |
| 3 | Waiting for T304 expiring. | - | - | - | - |
| 4 | The SS changes the power level setting according to the row "T2". | - | - | - | - |
| 5 | The UE transmits an RRCReestablishmentRequest message in NR Cell 1. | --> | NR RRC: RRCReestablishmentRequest | - | - |
| 6 | The SS transmits an RRCReestablishment message to resume SRB1 operation and re-activate security in NR Cell 1. | <-- | NR RRC: RRCReestablishment | - | - |
| 7 | Check: Does the UE transmit an RRCReestablishmentComplete message with rlf-InfoAvailable in NR Cell 1? | --> | NR RRC: RRCReestablishmentComplete | 1 | P |
| 8 | The SS transmits an RRCReconfiguration message to resume existing radio bearers on NR Cell 1. | <-- | NR RRC: RRCReconfiguration | - | - |
| 9 | The UE transmits an RRCReconfigurationtComplete message. | --> | NR RRC: RRCReconfigurationtComplete | - | - |
| 10 | The SS transmits a UEInformationRequest message. | <-- | NR RRC: UEInformationRequest | - | - |
| 11 | Check: Does the UE transmit a UEInformationResponse message including lastHO-Type-r17 set to daps and timeConnSourceDAPS-Failure-r17 in NR Cell 1? | --> | NR RRC: UEInformationResponse | 2 | P |

Table 8.1.6.1.3.10.3.2-4: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits preamble to NR Cell 2. | -> | (PRACH Preamble) | - | - |
| 2 | The SS does not respond. | - | - | - | - |

8.1.6.1.3.10.3.3	Specific message contents

Table 8.1.6.1.3.10.3.3-1: SIB1 for NR Cell 1 (all steps)

| Derivation Path: TS 38.508-1 [4] table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| ue-TimersAndConstants SEQUENCE { |  |  |  |
| t310 | ms0 | 0 ms |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.10.3.3-2: RRCReconfiguration (step 1, Table 8.1.6.1.3.10.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.8.1-1A with Condition RBConfig_KeyChange |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig with conditions (SRB_NR_PDCP) and (DRBn AND DAPS_PDCP) |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig)Table 8.1.6.1.3.10.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.10.3.3-3: CellGroupConfig (Table 8.1.6.1.3.10.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with condition PCell_change and CFRA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 3 entries |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions SRB1 | entry 1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions SRB2 | entry 2 |  |
| RLC-BearerConfig[3] | RLC-BearerConfig with conditions AM, DRBn | entry 3 |  |
| } |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| spCellConfigCommon SEQUENCE { |  |  |  |
| physCellId | Physical Cell Identity of NR Cell 2 |  |  |
| uplinkConfigCommon SEQUENCE { |  |  |  |
| initialUplinkBWP SEQUENCE { |  |  |  |
| rach-ConfigCommon CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| rach-ConfigGeneric | RACH-ConfigGeneric | Table 8.1.6.1.3.10.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.10.3.3-4: RACH-ConfigGeneric (Table 8.1.6.1.3.10.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| preambleTransMax | n200 |  |  |
| } |  |  |  |

Table 8.1.6.1.3.10.3.3-5: RRCReestablishment (step 6, Table 8.1.6.1.3.10.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReestablishment ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReestablishment SEQUENCE { |  |  |  |
| nextHopChainingCount | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.10.3.3-6: RRCReestablishmentComplete (step 7, Table 8.1.6.1.3.10.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReestablishmentComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ue-MeasurementsAvailable-r16 | UE-MeasurementsAvailable with condition RLF |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.10.3.3-7: RRCReconfiguration (step 8, Table 8.1.6.1.3.10.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition REEST

Table 8.1.6.1.3.10.3.3-8: UEInformationRequest (step 10, Table 8.1.6.1.3.10.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-32A with condition RLF

Table 8.1.6.1.3.10.3.3-9: UEInformationResponse (step 11, Table 8.1.6.1.3.10.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| rlf-Report-r16 CHOICE { |  |  |  |
| nr-RLF-Report-r16 SEQUENCE { |  |  |  |
| measResultLastServCell-r16 SEQUENCE { |  |  |  |
| measResult-r16 SEQUENCE { |  |  |  |
| cellResults-r16 SEQUENCE { |  |  |  |
| resultsSSB-Cell-r16 SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| resultsCSI-RS-Cell-r16 | Not checked |  |  |
| } |  |  |  |
| rsIndexResults-r16 SEQUENCE { |  |  |  |
| resultsSSB-Indexes-r16 SEQUENCE { |  |  |  |
| ssb-Index | 1 |  |  |
| ssb-Results SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| ssbRLMConfigBitmap-r16 | 01000000 00000000 00000000 00000000 00000000 00000000 00000000 00000000 |  |  |
| resultsCSI-RS-Indexes-r16 | Not present |  |  |
| csi-rsRLMConfigBitmap-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells-r16 | Not present |  |  |
| c-RNTI-r16 | the old value of the C-RNTI of the UE used in NR Cell 1 |  |  |
| previousPCellId-r16 CHOICE { |  |  |  |
| nrPreviousCell-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 | PLMN ID of NR Cell 1 |  |  |
| cellIdentity-r16 | CGI of NR Cell 1 |  |  |
| trackingAreaCode-r16 | TAC of NR Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| failedPCellId-r16 CHOICE { |  |  |  |
| nrFailedPCellId-r16 | Any Allowed Value | Either pci-arfcn-r16 or cellGlobalId-r16 corresponding to NR Cell 2 |  |
| } |  |  |  |
| reconnectCellId-r16 | Not present |  |  |
| timeUntilReconnection-16 | Not present |  |  |
| reestablishmentCellId-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 | PLMN ID of NR Cell 1 |  |  |
| cellIdentity-r16 | CGI of NR Cell 1 |  |  |
| trackingAreaCode-r16 | TAC of NR Cell 1 |  |  |
| } |  |  |  |
| timeConnFailure-r16 | Any allowed value |  |  |
| timeSinceFailure-r16 | Any allowed value |  |  |
| connectionFailureType-r16 | hof |  |  |
| rlf-Cause-r16 | Any allowed value |  |  |
| locationInfo-r16 | Not checked |  |  |
| noSuitableCellFound-r16 | Not present |  |  |
| ra-InformationCommon-r16 | Not checked |  |  |
| lastHO-Type-r17 | daps |  |  |
| timeConnSourceDAPS-Failure-r17 | (0..1023) |  |  |
| timeSinceCHO-Reconfig-r17 | Not present |  |  |
| choCellId-r17 | Not present |  |  |
| choCandidateCellList-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

###### 8.1.6.1.3.11	Radio Link Failure / Logging and reporting / Successful Handovers Reports

8.1.6.1.3.11.1	Test Purpose (TP)

(1)

with { UE having received RRC reconfiguration message and sourceDAPS-FailureReporting is included in the successHO-Config with value ‘true’, and having detected radio link failure in source PCell during the DAPS handover while T304 was running }

ensure that {

when { UE receives RRCReestablishment message }

then { UE sends the RRCReestablishmentComplete message with successHO-InfoAvailable }

}

(2)

with { UE having received RRC reconfiguration meesage and sourceDAPS-FailureReporting is included in the successHO-Config with value ‘true’, and having detected radio link failure in source PCell during the DAPS handover while T304 was running }

ensure that {

when { UE receives the UEInformationRequest message containing successHO-ReportReq }

then { UE sends the UEInformationResponse message with successHO-Report }

}

8.1.6.1.3.11.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331, clauses 5.3.7.5, 5.3.10.3, 5.3.5.8.3, 5.3.5.3, 5.7.10.3, and 5.7.10.6. Unless otherwise stated these are Rel-17 requirements.

[TS 38.331, clause 5.3.7.5]

The UE shall:

…

1>	set the content of RRCReestablishmentComplete message as follows:

…

2>	if the UE has successful handover information available in VarSuccessHO-Report and if the RPLMN is included in plmn-IdentityList stored in VarSuccessHO-Report:

3>	include successHO-InfoAvailable in the RRCReestablishmentComplete message;

1>	submit the RRCReestablishmentComplete message to lower layers for transmission;

[TS 38.331, clause 5.3.10.3]

The UE shall:

1>	if any DAPS bearer is configured and T304 is running:

2>	upon T310 expiry in source SpCell; or

2>	upon random access problem indication from source MCG MAC; or

2>	upon indication from source MCG RLC that the maximum number of retransmissions has been reached; or

2>	upon consistent uplink LBT failure indication from source MCG MAC:

3>	consider radio link failure to be detected for the source MCG i.e. source RLF;

3>	suspend the transmission and reception of all DRBs and multicast MRBs in the source MCG;

3>	reset MAC for the source MCG;

3>	release the source connection.

[TS 38.331, clause 5.3.5.8.3]

The UE shall:

1>	if T304 of the MCG expires:

2>	release dedicated preambles provided in rach-ConfigDedicated if configured;

2>	release dedicated msgA PUSCH resources provided in rach-ConfigDedicated if configured;

…

2>	else:

3>	revert back to the UE configuration used in the source PCell;

3>	store the handover failure information in VarRLF-Report as described in the subclause 5.3.10.5;

3>	initiate the connection re-establishment procedure as specified in subclause 5.3.7.

[TS 38.331, clause 5.3.5.3]

The UE shall perform the following actions upon reception of the RRCReconfiguration, or upon execution of the conditional reconfiguration (CHO or CPC):

…

1>	set the content of the RRCReconfigurationComplete message as follows:

…

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report; or

…

3>	include rlf-InfoAvailable in the RRCReconfigurationComplete message;

…

1>	else (RRCReconfiguration was received via SRB1):

2>	submit the RRCReconfigurationComplete message via SRB1 to lower layers for transmission using the new configuration;

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if the successHO-ReportReq is set to true and if the UE has successful handover related information available in VarSuccessHO-Report and if the RPLMN is included in the plmn-IdentityList stored in VarSuccessHO-Report:

2>	if the successHO-Report in the VarSuccessHO-Report concerns a DAPS handover and if a PDCP PDU has been received from the source cell of the concerned HO and a non-duplicated PDCP PDU has been received from the target cell of the concerned HO:

3>	set upInterruptionTimeAtHO in VarSuccessHO-Report to include the time elapsed between the time of arrival of the last PDCP PDU received from the source cell of the concerned handover and the time of arrival of the first non-duplicate PDCP PDU received from the target cell of the concerned handover, as measured at the time of arrival of the first non-duplicate PDCP PDU received from the target cell;

2>	set the successHO-Report in the UEInformationResponse message to the value of successHO-Report in the VarSuccessHO-Report, if available;

2>	discard the VarSuccessHO-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

…

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1.

[TS 38.331, clause 5.7.10.6]

The UE shall for the PCell:

…

1>	if sourceDAPS-FailureReporting is included in the successHO-Config before executing the last reconfiguration with sync and is set to true and if the last executed handover was a DAPS handover and if an RLF occurred at the source PCell during the DAPS handover while T304 was running:

2>	store the successful handover information in VarSuccessHO-Report and determine the content in VarSuccessHO-Report as follows:

3>	clear the information included in VarSuccessHO-Report, if any;

3>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e., includes the RPLMN);

3>	set the c-RNTI to the C-RNTI assigned by the target PCell of the handover;

3>	for the source PCell in which the last RRCReconfiguration message including reconfigurationWithSync was applied:

4>	set the sourceCellID in sourceCellInfo to the global cell identity and tracking area code, if available, of the source PCell;

4>	set the sourceCellMeas in sourceCellInfo to include the cell level RSRP, RSRQ and the available SINR, of the source PCell based on the available SSB and CSI-RS measurements collected up to the moment the UE sends RRCReconfigurationComplete message;

4>	set the rsIndexResults in sourceCellMeas to include all the available SSB and CSI-RS measurement quantities of the source PCell collected up to the moment the UE sends RRCReconfigurationComplete message;

4>	if the last executed handover was a DAPS handover and if an RLF occurred at the source PCell during the DAPS handover while T304 was running:

5>	set the rlf-InSourceDAPS in sourceCellInfo to true;

3>	for the target PCell indicated in the last applied RRCReconfiguration message including reconfigurationWithSync:

4>	set the targetCellID in targetCellInfo to the global cell identity and tracking area code, if available, of the target PCell;

4>	set the targetCellMeas in targetCellInfo to include the cell level RSRP, RSRQ and the available SINR, of the target PCell based on the available SSB and CSI-RS measurements collected up to the moment the UE sends RRCReconfigurationComplete message;

4>	set the rsIndexResults in targetCellMeas to include all the available SSB and CSI-RS measurement quantities of the target PCell collected up to the moment the UE sends RRCReconfigurationComplete message;

4>	if the last applied RRCReconfiguration message including reconfigurationWithSync was included in the stored condRRCReconfig:

5>	set the timeSinceCHO-Reconfig to the time elapsed between the initiation of the execution of conditional reconfiguration for the target PCell and the reception of the last conditionalReconfiguration including the condRRCReconfig of the target PCell in the source PCell;

3>	if the ratio between the value of the elapsed time of the timer T304 and the configured value of the T304 timer, included in the last applied RRCReconfiguration message including the reconfigurationWithSync, is greater than thresholdPercentageT304 if included in the successHO-Config received before executing the last reconfiguration with sync:

4>	set t304-cause in shr-Cause to true;

4>	set the ra-InformationCommon to include the random-access related information associated to the random access procedure in the target PCell, as specified in clause 5.7.10.5;

3>	if the ratio between the value of the elapsed time of the timer T310 and the configured value of the T310 timer, configured while the UE was connected to the source PCell before executing the last reconfiguration with sync, is greater than thresholdPercentageT310 included in the successHO-Config if configured by the source PCell before executing the last reconfiguration with sync:

4>	set t310-cause in shr-Cause to true;

3>	if the T312 associated to the measurement identity of the target cell was running at the time of initiating the execution of the reconfiguration with sync procedure and if the ratio between the value of the elapsed time of the timer T312 and the configured value of the T312 timer, configured while the UE was connected to the source PCell before executing the last reconfiguration with sync, is greater than thresholdPercentageT312 included in the successHO-Config if configured by the source PCell before executing the last reconfiguration with sync:

4>	set t312-cause in shr-Cause to true;

3>	if sourceDAPS-FailureReporting included in the successHO-Config if configured by the source PCell before executing the last reconfiguration with sync is set to true, and if the last executed handover was a DAPS handover and if an RLF occurred at the source PCell during the DAPS handover while T304 was running:

4>	set sourceDAPS-Failure in shr-Cause to true;

3>	for each of the measObjectNR, configured by the source PCell, in which the last RRCReconfiguration message including reconfigurationWithSync was applied:

4>	if measurements are available for the measObjectNR:

5>	if the SS/PBCH block-based measurement quantities are available:

6>	set the measResultListNR in measResultNeighCells to include all the available measurement quantities of the best measured cells, other than the source PCell or target PCell, ordered such that the cell with highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the cell with highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the cell with highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE sends the RRCReconfigurationComplete message;

6>	for each neighbour cell included, include the optional fields that are available;

NOTE 1:	For the neighboring cells set included in measResultListNR in measResultNeighCells ordered based on the SS/PBCH block measurement quantities, the UE includes also the CSI-RS based measurement quantities, if available.

5>	if the CSI-RS measurement quantities are available:

6>	set the measResultListNR in measResultNeighCells to include all the available measurement quantities of the best measured cells, other than the source PCell and target PCell, ordered such that the cell with highest CSI-RS RSRP is listed first if CSI-RS RSRP measurement results are available, otherwise the cell with highest CSI-RS RSRQ is listed first if CSI-RS RSRQ measurement results are available, otherwise the cell with highest CSI-RS SINR is listed first, based on the available CSI-RS based measurements collected up to the moment the UE sends the RRCReconfigurationComplete message;

6>	for each neighbour cell included, include the optional fields that are available;

NOTE 2:	For the neighboring cells set ordered based on the CSI-RS measurement quantities, the UE includes measurements only for the cells not yet included in measResultListNR in measResultNeighCells to avoid overriding SS/PBCH block-based ordered measurements.

3>	for each of the measObjectEUTRA, configured by the source PCell in which the last RRCReconfiguration message including reconfigurationWithSync was applied:

4>	if measurements are available for the measObjectEUTRA:

5>	set the measResultListEUTRA in measResultNeighCells to include the best measured cells ordered such that the cell with highest RSRP is listed first if RSRP measurement results are available, otherwise the cell with highest RSRQ is listed first, based on measurements collected up to the moment the UE sends the RRCReconfigurationComplete message;

5>	for each neighbour cell included, include the optional fields that are available;

3>	for each of the neighbour cells included in measResultNeighCells:

4>	if the cell was a candidate target cell included in the condRRCReconfig within the conditionalReconfiguration configured by the source PCell, in which the last RRCReconfiguration message including reconfigurationWithSync was applied:

5>	set the choCandidate to true in measResultNR;

3>	if available, set the locationInfo as in 5.3.3.7;

1>	release successHO-Config configured by the source PCell and thresholdPercentageT304 if configured by the target PCell.

The UE may discard the successful handover information, i.e., release the UE variable VarSuccessHO-Report, 48 hours after the last successful handover information is added to the VarSuccessHO-Report.

8.1.6.1.3.11.3	Test description

8.1.6.1.3.11.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the Serving cell and NR Cell 2 is the Suitable neighbour intra-frequency cell.

-	System information combination NR-2 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used for NR Cells.

UE:

-	None.

Preamble:

-	If pc_IP_Ping is set to TRUE then, the UE is in 5GS state 3N-A on NR Cell 1 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

-	Else, the UE is in 5GS state 3N-A and Test Loop Function (On) with UE test loop mode B on NR Cell 1 according to TS 38.508-1[4], clause 4.4A.2 Table 4.4A.2-3.

8.1.6.1.3.11.3.2	Test procedure sequence

Table 8.1.6.1.3.11.3.2-1 and Table 8.1.6.1.3.11.3.2-2 illustrate the downlink power levels and other changing parameters to be applied for the NR cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. The configuration "T0" indicates the initial conditions. Subsequent configurations marked "T1" and "T2" are applied at the points indicated in the Main behaviour description in Table 8.1.6.1.3.11.3.2-3.

Table 8.1.6.1.3.11.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -94 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | off | -94 |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -94 |  |

Table 8.1.6.1.3.11.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -91 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | off | -91 |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -91 |  |

Table 8.1.6.1.3.11.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message containing reconfigurationWithSync to order UE to perform DAPS handover to NR Cell 2. DRB#n of the first PDU session is configured as DAPS bearer. | <-- | NR RRC: RRCReconfiguration | - | - |
| - | EXCEPTION: In parallel with step 2-3, parallel behaviour defined in table 8.1.6.1.3.11.3.2-4 is executed. | - | - | - | - |
| 2 | The SS changes the power level setting according to the row "T1". | - | - | - | - |
| 3 | Waiting for T310 expiring. | - | - | - | - |
| 3A | The SS transmits Random Access Response to respond to the latest preamble on NR Cell 2. | <-- | Random Access Response | - | - |
| 3B | The UE transmits an RRCReconfigurationComplete message in NR Cell 2. | --> | NR RRC: RRCReconfigurationComplete | -- | -- |
| 4 | The SS changes the power level setting according to the row "T2".- | - | - | - | - |
| 5 | The UE transmits an RRCReestablishmentRequest message in NR Cell 1. | --> | NR RRC: RRCReestablishmentRequest | - | - |
| 6 | The SS transmits an RRCReestablishment message to resume SRB1 operation and re-activate security in NR Cell 1. | <-- | NR RRC: RRCReestablishment | - | - |
| 7 | Check: Does the UE transmit an RRCReestablishmentComplete message with successHO-InfoAvailable in NR Cell 1? | --> | NR RRC: RRCReestablishmentComplete | 1 | P |
| 8 | The SS transmits an RRCReconfiguration message to resume existing radio bearers on NR Cell 1. | <-- | NR RRC: RRCReconfiguration | - | - |
| 9 | The UE transmits an RRCReconfigurationtComplete message. | --> | NR RRC: RRCReconfigurationtComplete | - | - |
| 10 | The SS transmits an RRCReconfiguration message with condition DAPS_HO_ReleaseSource in NR Cell 2. | <-- | NR RRC: RRCReconfiguration | - | - |
| 11 | The UE transmits an RRCReconfigurationComplete message in NR Cell 2. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 12 | The SS transmits a UEInformationRequest message. | <-- | NR RRC: UEInformationRequest | - | - |
| 13 | Check: Does the UE transmit a UEInformationResponse message including successHO-Report in NR Cell 2? | --> | NR RRC: UEInformationResponse | 2 | P |

Table 8.1.6.1.3.11.3.2-4: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits preamble to NR Cell 2. | --> | (PRACH Preamble) | - | - |
| 2 | The SS does not respond. | - | - | - | - |

8.1.6.1.3.11.3.3	Specific message contents

Table 8.1.6.1.3.11.3.3-1: SIB1 for NR Cell 1 (all steps)

| Derivation Path: TS 38.508-1 [4] table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| ue-TimersAndConstants SEQUENCE { |  |  |  |
| t310 | ms50 | 50 ms |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.11.3.3-2: RRCReconfiguration (step 1, Table 8.1.6.1.3.11.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.8.1-1A with Condition RBConfig_KeyChange |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig with conditions (SRB_NR_PDCP) and (DRBn AND DAPS_PDCP) |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig)Table 8.1.6.1.3.11.3.3-3 |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| otherConfig-v1700 SEQUENCE { |  |  |  |
| successHO-Config-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sourceDAPS-FailureReporting-r17 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.11.3.3-3: CellGroupConfig (Table 8.1.6.1.3.11.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with condition PCell_change and CFRA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 3 entries |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions SRB1 | entry 1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions SRB2 | entry 2 |  |
| RLC-BearerConfig[3] | RLC-BearerConfig with conditions AM, DRBn | entry 3 |  |
| } |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| spCellConfigCommon SEQUENCE { |  |  |  |
| physCellId | Physical Cell Identity of NR Cell 2 |  |  |
| uplinkConfigCommon SEQUENCE { |  |  |  |
| initialUplinkBWP SEQUENCE { |  |  |  |
| rach-ConfigCommon CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| rach-ConfigGeneric | RACH-ConfigGeneric | Table 8.1.6.1.3.11.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.11.3.3-4: RACH-ConfigGeneric (Table 8.1.6.1.3.11.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| preambleTransMax | n200 |  |  |
| } |  |  |  |

able 8.1.6.1.3.11.3.3-5: RRCReestablishment (step 6, Table 8.1.6.1.3.11.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReestablishment ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReestablishment SEQUENCE { |  |  |  |
| nextHopChainingCount | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.11.3.3-6: RRCReestablishmentComplete (step 7, Table 8.1.6.1.3.11.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReestablishmentComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ue-MeasurementsAvailable-r16 | UE-MeasurementsAvailable with condition SHR |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.11.3.3-7: RRCReconfiguration (step 8, Table 8.1.6.1.3.11.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition REEST

Table 8.1.6.1.3.11.3.3-8: RRCReconfiguration (step 10, Table 8.1.6.1.3.11.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with Condition DAPS_HO_ReleaseSource

Table 8.1.6.1.3.11.3.3-9: UEInformationRequest (step 12, Table 8.1.6.1.3.11.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-32A with condition SHR

Table 8.1.6.1.3.11.3.3-10: UEInformationResponse (step 13, Table 8.1.6.1.3.11.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| nonCriticalExtension CHOICE { |  |  |  |
| successHO-Report-r17 SEQUENCE { |  |  |  |
| sourceCellInfo-r17 SEQUENCE { |  |  |  |
| sourcePCellId-r17 SEQUENCE { |  |  |  |
| plmn-Identity-r16 | PLMN ID of NR Cell 1 |  |  |
| cellIdentity-r16 | CGI of NR Cell 1 |  |  |
| trackingAreaCode-r16 | TAC of NR Cell 1 |  |  |
| } |  |  |  |
| sourceCellMeas-r17 SEQUENCE { |  |  |  |
| measResult-r17 SEQUENCE { |  |  |  |
| cellResults-r17 SEQUENCE { |  |  |  |
| resultsSSB-Cell-r17 SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| resultsCSI-RS-Cell-r16 | Not checked |  |  |
| } |  |  |  |
| rsIndexResults-r17 SEQUENCE { |  |  |  |
| resultsSSB-Indexes-r17 SEQUENCE { |  |  |  |
| ssb-Index | 1 |  |  |
| ssb-Results SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| resultsCSI-RS-Indexes-r17 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| rlf-InSourceDAPS-r17 | true |  |  |
| } |  |  |  |
| targetCellInfo-r17 SEQUENCE { |  |  |  |
| targetPCellId-r17 SEQUENCE { |  |  |  |
| plmn-Identity-r16 | PLMN ID of NR Cell 2 |  |  |
| cellIdentity-r16 | CGI of NR Cell 2 |  |  |
| trackingAreaCode-r16 | TAC of NR Cell 2 |  |  |
| } |  |  |  |
| targetCellMeas-r17 SEQUENCE { |  |  |  |
| measResult-r17 SEQUENCE { |  |  |  |
| cellResults-r17 SEQUENCE { |  |  |  |
| resultsSSB-Cell-r16 SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| resultsCSI-RS-Cell-r16 | Not checked |  |  |
| } |  |  |  |
| rsIndexResults-r17 SEQUENCE { |  |  |  |
| resultsSSB-Indexes-r16 SEQUENCE { |  |  |  |
| ssb-Index | 1 |  |  |
| ssb-Results SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| resultsCSI-RS-Indexes-r17 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells-r17 | Not present |  |  |
| locationInfo-r17 | Not checked |  |  |
| timeSinceCHO-Reconfig-r17 | Any allowed value |  |  |
| shr-Cause-r17 SEQUENCE { |  |  |  |
| sourceDAPS-Failure-r17 | true |  |  |
| } |  |  |  |
| ra-InformationCommon-r17 | Not checked |  |  |
| upInterruptionTimeAtHO-r17 | (0..1023) |  |  |
| c-RNTI-r17 | the value of the C-RNTI of the UE used in NR Cell 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 8.1.6.1.4	Connection Establishment Failure

###### 8.1.6.1.4.1	Connection Establishment Failure / Logging and reporting / T300 expiry

8.1.6.1.4.1.1	Test Purpose (TP)

(1)

with { UE having sent an RRCSetupRequest message }

ensure that {
  when { T300 is expired and random access procedure failed }

then { UE stores the connection establishment failure information }

}

(2)

with { UE having sent an RRCSetupComplete message with connEstFailInfoAvailable }

ensure that {

when { UE receives a UEInformationRequest message with connEstFailReportReq set to true }

then { UE sends a UEInformationResponse message with connEstFailReport }

}

8.1.6.1.4.1.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 38.331 clause 5.3.3.4, 5.3.3.7, 5.7.10.3.[TS 38.331, clause 5.3.3.4]

The UE shall perform the following actions upon reception of the RRCSetup:

…1>	set the content of RRCSetupComplete message as follows:

…

2>	if the UE has connection establishment failure or connection resume failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

3>	include connEstFailInfoAvailable in the RRCSetupComplete message;

…

[TS 38.331, clause 5.3.3.7]

The UE shall:

1>	if timer T300 expires:

2>	reset MAC, release the MAC configuration and re-establish RLC for all RBs that are established;

2>	if the UE supports RRC Connection Establishment failure with temporary offset and the T300 has expired a consecutive connEstFailCount times on the same cell for which connEstFailureControl is included in SIB1:

3>	for a period as indicated by connEstFailOffsetValidity:

4>	use connEstFailOffset for the parameter Qoffsettemp for the concerned cell when performing cell selection and reselection according to TS 38.304 [20] and TS 36.304 [27];

NOTE 1:	When performing cell selection, if no suitable or acceptable cell can be found, it is up to UE implementation whether to stop using connEstFailOffset for the parameter Qoffsettemp during connEstFailOffsetValidity for the concerned cell.

2>	if the UE has connection establishment failure informaton or connection resume failure information available in VarConnEstFailReport and if the RPLMN is not equal to plmn-identity stored in VarConnEstFailReport; or

2>	if the cell identity of current cell is not equal to the cell identity stored in measResultFailedCell in VarConnEstFailReport:

3>	reset the numberOfConnFail to 0;

2>	clear the content included in VarConnEstFailReport except for the numberOfConnFail, if any;

2>	store the following connection establishment failure information in the VarConnEstFailReport by setting its fields as follows:

3>	set the plmn-Identity to the PLMN selected by upper layers (see TS 24.501 [23]) from the PLMN(s) included in the plmn-IdentityList in SIB1;

3>	set the measResultFailedCell to include the global cell identity, tracking area code, the cell level and SS/PBCH block level RSRP, and RSRQ, and SS/PBCH block indexes, of the failed cell based on the available SSB measurements collected up to the moment the UE detected connection establishment failure;

3>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements for at most the following number of neighbouring cells: 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies per RAT and according to the following:

4>	for each neighbour cell included, include the optional fields that are available;

NOTE 2:	The UE includes the latest results of the available measurements as used for cell reselection evaluation, which are performed in accordance with the performance requirements as specified in TS 38.133 [14].

3>	if available, set the locationInfo as follows:

4>	if available, set the commonLocationInfo to include the detailed location information;

4>	if available, set the bt-LocationInfo to include the Bluetooth measurement results, in order of decreasing RSSI for Bluetooth beacons;

4>	if available, set the wlan-LocationInfo to include the WLAN measurement results, in order of decreasing RSSI for WLAN APs;

4>	if available, set the sensor-LocationInfo to include the sensor measurement results as follows;

5>	if available, include the sensor-MeasurementInformation;

5>	if available, include the sensor-MotionInformation;

3>	set perRAInfoList to indicate random access failure information as specified in 5.7.10.5;

3>	if the numberOfConnFail is smaller than 8:

4>	increment the numberOfConnFail by 1;

2>	inform upper layers about the failure to establish the RRC connection, upon which the procedure ends;

The UE may discard the connection establishment failure or connection resume failure information, i.e. release the UE variable VarConnEstFailReport, 48 hours after the last connection establishment failure is detected.

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if connEstFailReportReq is set to true and the UE has connection establishment failure or connection resume failure information in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

2>	set timeSinceFailure in VarConnEstFailReport to the time that elapsed since the last connection establishment failure or connection resume failure in NR;

2>	set the connEstFailReport in the UEInformationResponse message to the value of connEstFailReport in VarConnEstFailReport;

2>	discard the connEstFailReport from VarConnEstFailReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

…

8.1.6.1.4.1.3	Test description

8.1.6.1.4.1.3.1	Pre-test conditions

System Simulator:

- NR Cell 1.

- System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.2.

UE:

- None.

Preamble:

- The UE is in state 1N-A as defined in TS 38.508-1 [4], subclause 4.4A.

8.1.6.1.4.1.3.2	Test procedure sequence

Table 8.1.6.1.4.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a Paging message. | <-- | NR RRC: Paging | - | - |
| 2 | The UE transmits an RRCSetupRequest message. | --> | NR RRC: RRCSetupRequest | - | - |
| 3 | The SS waits for 2sec to ensure that the UE detects T300 expiry. | - | - | - | - |
| 4 | The SS transmits a Paging message. | <-- | NR RRC: Paging | - | - |
| 5 | The UE transmits an RRCSetupRequest message. | --> | NR RRC: RRCSetupRequest | - | - |
| 6 | The SS transmits an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 7 | Check: Does the UE transmit an RRCSetupComplete message including connEstFailInfoAvailable IE set it to true? UE initiates the session management procedure by including the SERVICE REQUEST message. | --> | NR RRC: RRCSetupComplete 5GMM: SERVICE REQUEST | 1 | P |
| 8-11 | Steps 5 to 8 of the generic radio bearer establishment procedure in TS 38.508-1 table 4.5.4.2-3 | - | - | - | - |
| 12 | The SS sends a UEInformationRequest message to get connEstFailReportReq. | <-- | NR RRC: UEInformationRequest | - | - |
| 13 | Check: Does the UE send a UEInformationResponse message with connEstFailReport? | --> | NR RRC: UEInformationResponse | 2 | P |

8.1.6.1.4.1.3.3	Specific message contents

Table 8.1.6.1.4.1.3.3-1: RRCSetupComplete (step 7, Table 8.1.6.1.4.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ue-MeasurementsAvailable-r16 | UE-MeasurementsAvailable-r16 with condition CEF |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.4.1.3.3-2: Void

Table 8.1.6.1.4.1.3.3-3: UEInformationResponse (step 13, Table 8.1.6.1.4.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| connEstFailReport-r16 SEQUENCE { |  |  |  |
| measResultFailedCell-r16 SEQUENCE { |  |  |  |
| cgi-Info SEQUENCE { |  |  |  |
| plmn-Identity-r16 | plmn-Identity within SIB1 broadcasted in NR Cell 1 |  |  |
| cellIdentity-r16 | cellIdentity within SIB1 broadcasted in NR Cell 1 | BIT STRING (SIZE (36)) |  |
| trackingAreaCode-r16 | Not present or any allowed value | BIT STRING (SIZE (24)) |  |
| } |  |  |  |
| measResult-r16 SEQUENCE { |  |  |  |
| cellResults-r16 | Not present or any allowed value |  |  |
| rsIndexResults-r16 | Not present or any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| locationInfo-r16 | Not present or any allowed value |  |  |
| measResultNeighCells-r16 SEQUENCE { |  |  |  |
| measResultNeighCellListNR | Not present |  |  |
| measResultNeighCellListEUTRA | Not present |  |  |
| } |  |  |  |
| numberOfConnFail-r16 | Any allowed value | INTEGER (1..8) |  |
| perRAInfoList-r16 | Any allowed value |  |  |
| timeSinceFailure-r16 | Any allowed value | INTEGER (0..172800) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

###### 8.1.6.1.4.2	Connection Establishment Failure / Logging and reporting / RRC Resume

8.1.6.1.4.2.1	Test Purpose (TP)

(1)

with { UE having sent an RRCResumeRequest message }

ensure that {
  when { T319 is expired and random access procedure failed }

then { UE stores the connection resume failure information }

}

(2)

with { UE having sent an RRCSetupComplete message with connEstFailInfoAvailable }

ensure that {

when { UE receives a UEInformationRequest message with connEstFailReportReq set to true }

then { UE sends a UEInformationResponse message with connEstFailReport }

}

8.1.6.1.4.2.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 38.331 clause 5.3.3.4, 5.3.13.5, 5.7.10.3.

[TS 38.331, clause 5.3.3.4]

The UE shall perform the following actions upon reception of the RRCSetup:

…1>	set the content of RRCSetupComplete message as follows:

…

2>	if the UE has connection establishment failure or connection resume failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

3>	include connEstFailInfoAvailable in the RRCSetupComplete message;

…

[TS 38.331, clause 5.3.13.5]

The UE shall:

1>	if timer T319 expires or upon receiving Integrity check failure indication from lower layers while T319 is running:

2>	if the UE has connection establishment failure information or connection resume failure informaton available in VarConnEstFailReport and if the RPLMN is not equal to plmn-identity stored in VarConnEstFailReport; or

2>	if the cell identity of current cell is not equal to the cell identity stored in measResultFailedCell in VarConnEstFailReport:

3>	reset the numberOfConnFail to 0;

2> clear the content included in VarConnEstFailReport except for the numberOfConnFail, if any;

2>	store the following connection resume failure information in the VarConnEstFailReport by setting its fields as follows:

3>	set the plmn-Identity to the PLMN selected by upper layers (see TS 24.501 [23]) from the PLMN(s) included in the plmn-IdentityList in SIB1;

3>	set the measResultFailedCell to include the global cell identity, tracking area code, the cell level and SS/PBCH block level RSRP, and RSRQ, and SS/PBCH block indexes, of the failed cell based on the available SSB measurements collected up to the moment the UE detected connection resume failure;

3>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements for at most the following number of neighbouring cells: 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies per RAT and according to the following:

4>	for each neighbour cell included, include the optional fields that are available;

NOTE:	The UE includes the latest results of the available measurements as used for cell reselection evaluation, which are performed in accordance with the performance requirements as specified in TS 38.133 [14].

3>	if available, set the locationInfo as in 5.3.3.7;

3>	set perRAInfoList to indicate random access failure information as specified in 5.7.10.5;

3>	if numberOfConnFail is smaller than 8:

4>	increment the numberOfConnFail by 1;

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause 'RRC Resume failure'.

The UE may discard the connection resume failure or connection establishment failure information, i.e. release the UE variable VarConnEstFailReport, 48 hours after the last connection resume failure is detected.

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if connEstFailReportReq is set to true and the UE has connection establishment failure or connection resume failure information in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

2>	set timeSinceFailure in VarConnEstFailReport to the time that elapsed since the last connection establishment failure or connection resume failure in NR;

2>	set the connEstFailReport in the UEInformationResponse message to the value of connEstFailReport in VarConnEstFailReport;

2>	discard the connEstFailReport from VarConnEstFailReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

…

[TS 24.501, clause 5.3.1.4]

…

If the UE in 5GMM-CONNECTED mode with RRC inactive indication receives an indication from the lower layers that the resumption of the RRC connection has failed, and:

a)	if the lower layers indicate that access barring is applicable for all access categories except categories 0 and 2, the UE shall:

1)	stay in 5GMM-CONNECTED mode with RRC inactive indication;

b)	else, the UE shall:

1)	enter 5GMM-IDLE mode; and

2)	initiate the registration procedure for mobility and periodic registration update used for mobility (i.e. the 5GS registration type IE set to "mobility registration updating" in the REGISTRATION REQUEST message) for N1 NAS signalling connection recovery as specified in subclause 5.5.1.3.2.

NOTE 2:	An indication from the lower layer that the RRC connection has been released with cause "RRC resume failure" can be considered as an indication that the resumption of the RRC connection has failed.

8.1.6.1.4.2.3	Test description

8.1.6.1.4.2.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

UE:

-	None.

Preamble:

-	The UE is in state 2N-A on NR Cell 1 as defined in TS 38.508-1 [4], subclause 4.4A.

8.1.6.1.4.2.3.2	Test procedure sequence

Table 8.1.6.1.4.2.3.2-1: Void

Table 8.1.6.1.4.2.3.2-2: Void

Table 8.1.6.1.4.2.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a Paging message on NR Cell 1. | <-- | NR RRC: Paging | - | - |
| 2 | The UE transmits an RRCResumeRequest message. | --> | NR RRC: RRCResumeRequest | - | - |
| 3 | The SS waits for 1 sec to ensure that the UE detects T319 expiry. | - | - | - | - |
| 3A | The UE transmits an RRCSetupRequest message. | --> | NR RRC: RRCSetupRequest |  |  |
| 3B | The SS transmits an RRCSetup message. | <-- | NR RRC: RRCSetup |  |  |
| 3C | Check: Does the UE transmit an RRCSetupComplete message including connEstFailInfoAvailable IE set it to true? UE initiates the mobility registration procedure by including the REGISTRATION REQUEST message. | --> | NR RRC: RRCSetupComplete 5GMM: REGISTRATION REQUEST | 1 | P |
| 3D-3E | Steps4-5 defined in TS 38.508-1 [4] Table 4.9.5.2.2-1, indicate that the UE has performed mobility registration on NR cell 1 started in NR RRC_IDLE? | - | - | - | - |
| 4 | The SS transmits a SecurityModeCommand message. | <-- | NR RRC: SecurityModeCommand | - | - |
| 5 | The UE transmits a SecurityModeComplete message. | --> | NR RRC: SecurityModeComplete | - | - |
| 6-12 | Void |  |  |  |  |
| 13 | The SS sends a UEInformationRequest message to get connEstFailReportReq. | <-- | NR RRC: UEInformationRequest | - | - |
| 14 | Check: Does the UE send a UEInformationResponse message with connEstFailReport? | --> | NR RRC: UEInformationResponse | 2 | P |

8.1.6.1.4.2.3.3	Specific message contents

Table 8.1.6.1.4.2.3.3-1: RRCSetupComplete (step 3C, Table 8.1.6.1.4.2.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ue-MeasurementsAvailable-r16 | UE-MeasurementsAvailable-r16 with condition CEF |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.4.2.3.3-2: Void

Table 8.1.6.1.4.2.3.3-3: UEInformationResponse (step 14, Table 8.1.6.1.4.2.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| connEstFailReport-r16 SEQUENCE { |  |  |  |
| measResultFailedCell-r16 SEQUENCE { |  |  |  |
| cgi-Info SEQUENCE { |  |  |  |
| plmn-Identity-r16 | plmn-Identity within SIB1 broadcasted in NR Cell 1 |  |  |
| cellIdentity-r16 | cellIdentity within SIB1 broadcasted in NR Cell 1 | BIT STRING (SIZE (36)) |  |
| trackingAreaCode-r16 | Not checked | BIT STRING (SIZE (24)) |  |
| } |  |  |  |
| measResult-r16 SEQUENCE { |  |  |  |
| cellResults-r16 | Not checked |  |  |
| rsIndexResults-r16 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| locationInfo-r16 | Not checked |  |  |
| measResultNeighCells-r16 SEQUENCE { |  |  |  |
| measResultNeighCellListNR | Not present |  |  |
| measResultNeighCellListEUTRA | Not present |  |  |
| } |  |  |  |
| numberOfConnFail-r16 | Not checked | INTEGER (1..8) |  |
| perRAInfoList-r16 | Not checked |  |  |
| timeSinceFailure-r16 | Not checked | INTEGER (0..172800) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

###### 8.1.6.1.4.3	Connection Establishment Failure / Logging and reporting / Reporting at intra-NR handover

8.1.6.1.4.3.1	Test Purpose (TP)

(1)

with { UE has connection establishment failure information available }

ensure that {

when { UE performs an intra-NR Handover procedure }

then { UE sends an RRCReconfigurationComplete message with connEstFailInfoAvailable }

}

8.1.6.1.4.3.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clauses 5.3.5.3. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.3.5.3]

The UE shall perform the following actions upon reception of the RRCReconfiguration, or upon execution of the conditional reconfiguration (CHO or CPC):

…

1>	set the content of the RRCReconfigurationComplete message as follows:

…

2>	if the UE has connection establishment failure or connection resume failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

3>	include connEstFailInfoAvailable in the RRCReconfigurationComplete message;

8.1.6.1.4.3.3	Test description

8.1.6.1.4.3.3.1	Pre-test conditions

System Simulator:

- NR Cell 1 and NR Cell 3.

- System information combination NR-4 as defined in TS 38.508-1 [4] clause 4.4.3.1.2.

UE:

- None.

Preamble:

- The UE is in state 1N-A as defined in TS 38.508-1 [4], subclause 4.4A.

8.1.6.1.4.3.3.2	Test procedure sequence

Table 8.1.6.1.4.3.3.2-1/2 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.6.1.4.3.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -85 | -97 | Power levels are such that entry condition for event A3 is not satisfied for the neighbour NR cell. |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -85 | -78 | Power levels are such that entry condition for event A3 is satisfied for the neighbour NR cell. |

Table 8.1.6.1.4.3.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -91 | -100 | Power levels are such that entry condition for event A3 is not satisfied for the neighbour NR cell. |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -91 | -82 | Power levels are such that entry condition for event A3 is satisfied for the neighbour NR cell. |

Table 8.1.6.1.4.3.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Void | - | - | - | - |
| 2 | The SS transmits a Paging message. | <-- | NR RRC: Paging | - | - |
| 3 | The UE transmits an RRCSetupRequest message. | --> | NR RRC: RRCSetupRequest | - | - |
| 4 | The SS waits for 2 sec to ensure that the UE detects T300 expiry. | - | - | - | - |
| 5 | Void | - | - | - | - |
| 6 | The SS transmits a Paging message. | <-- | NR RRC: Paging | - | - |
| 7-8 | Steps 2 to 3 of the generic radio bearer establishment procedure in TS 38.508-1 table 4.5.4.2-3 on NR Cell 1. | - | - | - | - |
| 9 | The UE transmits an RRCSetupComplete message and a SERVICE REQUEST message. | --> | NR RRC: RRCSetupComplete 5GMM: SERVICE REQUEST | - | - |
| 10-13 | Steps 5 to 8 of the generic radio bearer establishment procedure in TS 38.508-1 table 4.5.4.2-3 on NR Cell 1. | - | - | - | - |
| 14 | The SS transmits an RRCReconfiguration message to configure measurement. | <-- | NR RRC: RRCReconfiguration | - | - |
| 15 | UE transmit an RRCReconfigurationComplete message to confirm the measurement configuration. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 16 | The SS changes NR Cell 1 and NR Cell 3 parameters according to the row "T1"in table 8.1.6.1.4.3.3.2-1/2. | - | - | - | - |
| 17 | The UE transmits a MeasurementReport message. | --> | NR RRC: MeasurementReport | - | - |
| 18 | The SS transmits an RRCReconfiguration message to order the UE to perform handover to NR Cell 3. | <-- | NR RRC: RRCReconfiguration | - | - |
| 19 | Check: Dose the UE transmit an RRCReconfigurationComplete message containing connEstFailInfoAvailable on NR Cell 3? | --> | NR RRC: RRCReconfigurationComplete | 1 | P |
| 20 | The SS sends a UEInformationRequest message to get connEstFailReport. | <-- | NR RRC: UEInformationRequest | - | - |
| 21 | The UE send a UEInformationResponse message with connEstFailReport. | --> | NR RRC: UEInformationResponse | - | - |

8.1.6.1.4.3.3.3	Specific message contents

Table 8.1.6.1.4.3.3.3-1: Void

Table 8.1.6.1.4.3.3.3-1A: RRCSetupComplete (step 9, Table 8.1.6.1.4.3.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ue-MeasurementsAvailable-r16 | UE-MeasurementsAvailable-r16 with condition CEF |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.4.3.3.3-2: RRCReconfiguration (step 14, Table 8.1.6.1.4.3.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-13, condition MEAS

Table 8.1.6.1.4.3.3.3-3: MeasConfig (Table 8.1.6.1.4.3.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF MeasObjectToAddMod { | 2 entries |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR SEQUENCE { |  |  |  |
| ssbFrequency | ssbFrequency IE equals the ARFCN for NR Cell 3 |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measObjectId | 2 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR SEQUENCE { |  |  |  |
| ssbFrequency | ssbFrequency IE equals the ARFCN for NR Cell 1 |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-EventA3 |  |  |
| } |  |  |  |
| } |  |  |  |
| MeasIdToAddModList::= SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.4.3.3.3-3A: ReportConfigNR-EventA3 (Table 8.1.6.1.4.3.3.3-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-142 with condition EVENT_A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| Rsrp | 2 | 1 dB (2*0.5 dB) | FR1 |
|  | 2+ Delta(NRf2) - Delta(NRf1) | a3-Offset value set to (2+ Delta(NRf2) - Delta(NRf1)). (Note 1) | FR2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportAmount | r1 |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| Rsrp | true |  |  |
| Rsrq | false |  |  |
| Sinr | false |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| Note 1: Delta(NRf2) and Delta(NRf1) are derived based on calibration procedure defined in the TS 38.508-1 [4], clause 6.1.3.3 where Delta(NRf1) is derived for the carrier of NR PCell and Delta(NRf2) is derived for the carrier of neighbour cell. |  |  |  |

Table 8.1.6.1.4.3.3.3-4: MeasurementReport (step 17, Table 8.1.6.1.4.3.3.3-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-5A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions ::= CHOICE { |  |  |  |
| measurementReport ::= SEQUENCE { |  |  |  |
| measResults ::= SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServingMOList SEQUENCE (SIZE (1.. maxNrofServingCells)) OF MeasResultServMO { | 1 entry |  |  |
| MeasResultServMO[1] SEQUENCE { |  | entry 1 |  |
| servCellId | Cell index corresponding to NR Cell 1 |  |  |
| measResultServingCell SEQUENCE { |  |  |  |
| physCellId | PhysicalCellIdentity of NR Cell 1 |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListNR SEQUENCE (SIZE (1.. maxCellReport)) OF MeasResultNR { | 1 entry |  |  |
| MeasResultNR[1] SEQUENCE { |  |  |  |
| physCellId | Physical layer cell identity of NR Cell 3 |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | Not present |  |  |
| sinr | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.4.3.3.3-5: RRCReconfiguration (step 18, Table 8.1.6.1.4.3.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.8.1-1A, condition RBConfig_NoKeyChange

Table 8.1.6.1.4.3.3.3-6: RRCReconfigurationComplete (step 19, Table 8.1.6.1.4.3.3.2-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfigurationComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfigurationComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ue-MeasurementsAvailable-r16 | UE-MeasurementsAvailable-r16 with condition CEF |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

###### 8.1.6.1.4.4	Connection Establishment Failure / Logging and reporting / Reporting at RRC connection re-establishment

8.1.6.1.4.4.1	Test Purpose (TP)

(1)

with { UE has connection establishment failure information available }

ensure that {

when { UE performs an RRC Connection re-establishment procedure }

then { UE sends an RRCReestablishmentComplete message with connEstFailInfoAvailable }

}

8.1.6.1.4.4.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clauses 5.3.7.5. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.3.7.5]

The UE shall:

…

1>	set the content of RRCReestablishmentComplete message as follows:

2>	if the UE has logged measurements available for NR and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable in the RRCReestablishmentComplete message;

…

2>	if the UE has connection establishment failure or connection resume failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

3>	include connEstFailInfoAvailable in the RRCReestablishmentComplete message;

8.1.6.1.4.4.3	Test description

8.1.6.1.4.4.3.1	Pre-test conditions

System Simulator:

- NR Cell 1 and Cell 2.

- System information combination NR-2 as defined in TS 38.508-1 [4] clause 4.4.3.1.2.

UE:

- None.

Preamble:

- The UE is in state 1N-A as defined in TS 38.508-1 [4], subclause 4.4A.

8.1.6.1.4.4.3.2	Test procedure sequence

Table 8.1.6.1.4.4.3.2-1/2 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.6.1.4.4.3.2-1: Time instances of cell power level and parameter changes (FR1)

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -80 | ”Off” | Only NR Cell 1 is available. (NOTE 1). |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | ”Off” | -80 | Only NR Cell 2 is available. (NOTE 1). |
| NOTE 1: Power level “Off” corresponds to “Non-suitable "Off" cell” as defined in TS 38.508-1[4] Table 6.2.2.1-3. |  |  |  |  |  |

Table 8.1.6.1.4.4.3.2-2: Time instances of cell power level and parameter changes (FR2)

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -82 | ”Off” | Only NR Cell 1 is available. (NOTE 1). |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | ”Off” | -82 | Only NR Cell 2 is available. (NOTE 1). |
| NOTE 1: Power level “Off” corresponds to “Non-suitable "Off" cell” as defined in TS 38.508-1[4] Table 6.2.2.2.2-2. |  |  |  |  |  |

Table 8.1.6.1.4.4.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a Paging message. | <-- | NR RRC: Paging | - | - |
| 2 | The UE transmits an RRCSetupRequest message. | --> | NR RRC: RRCSetupRequest | - | - |
| 3 | The SS waits for 2 sec to ensure that the UE detects T300 expiry. | - | - | - | - |
| 4-6 | Steps 1 to 3 of the generic radio bearer establishment procedure in TS 38.508-1 table 4.5.4.2-3 on NR Cell 1. | - | - | - | - |
| 7 | The UE transmits an RRCSetupComplete message and a SERVICE REQUEST message. | --> | NR RRC: RRCSetupComplete | - | - |
| 8-11 | Steps 5 to 8 of the generic radio bearer establishment procedure in TS 38.508-1 table 4.5.4.2-3 on NR Cell 1. | - | - | - | - |
| 12 | The SS changes Cell 1 and Cell 2 parameters according to the row "T1" in table 8.1.6.1.4.4.3.2-1/2. | - |  | - | - |
| 13 | The UE transmits an RRCReestablishmentRequest message on Cell 2. | --> | NR RRC: RRCReestablishmentRequest | - | - |
| 14 | The SS transmits an RRCReestablishment message on Cell 2. | <-- | NR RRC: RRCReestablishment | - | - |
| 15 | Check: Does the UE send an RRCReestablishmentComplete message with connEstFailInfoAvailable on Cell 2? | --> | NR RRC: RRCReestablishmentComplete | 1 | P |
| 16 | The SS transmits an RRCReconfiguration message to establish SRB2 and DRB. | <-- | NR RRC: RRCReconfiguration | - | - |
| 17 | The UE transmits an RRCReconfigurationtComplete message. | --> | NR RRC: RRCReconfigurationtComplete | - | - |
| 18 | The SS sends a UEInformationRequest message to get connEstFailReport and rlf-Report. | <-- | NR RRC: UEInformationRequest | - | - |
| 19 | The UE send a UEInformationResponse message with connEstFailReport and rlf-Report. | --> | NR RRC: UEInformationResponse | - | - |

8.1.6.1.4.4.3.3	Specific message contents

Table 8.1.6.1.4.4.3.3-1: RRCSetupComplete (step 7, Table 8.1.6.1.4.4.3.2-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcSetupComplete SEQUENCE{ |  |  |  |
| nonCriticalExtension EQUENCE{ |  |  |  |
| ue-MeasurementsAvailable-r16 | UE-MeasurementsAvailable-r16 with condition CEF |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.4.4.3.3-2: RRCReestablishmentRequest (step 13, Table 8.1.6.1.4.4.3.2-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReestablishmentRequest ::= SEQUENCE { |  |  |  |
| reestablishmentCause | otherFailure |  |  |

Table 8.1.6.1.4.4.3.3-3: RRCReestablishmentComplete (step 15, Table 8.1.6.1.4.4.3.2-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReestablishmentComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ue-MeasurementsAvailable-r16 | UE-MeasurementsAvailable-r16 with condition CEF and RLF |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.4.4.3.3-4: UEInformationRequest (step 18, Table 8.1.6.1.4.4.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-32A with condition RLF

###### 8.1.6.1.4.5	Connection Establishment Failure / Logging and reporting / Location Information

8.1.6.1.4.5.1	Test Purpose (TP)

(1)

with { UE has connection establishment failure information available with location information }

ensure that {

when { UE receives the UEInformationRequest message containing connEstFailReportReq }

then { UE sends the UEInformationResponse message containing connEstFailReport with CommonLocationInfo }

}

8.1.6.1.4.5	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clauses 5.3.3.7, 5.7.10.3 and 6.3.2. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if connEstFailReportReq is set to true and the UE has connection establishment failure or connection resume failure information in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

2>	set timeSinceFailure in VarConnEstFailReport to the time that elapsed since the last connection establishment failure or connection resume failure in NR;

2>	set the connEstFailReport in the UEInformationResponse message to the value of connEstFailReport in VarConnEstFailReport;

2>	discard the connEstFailReport from VarConnEstFailReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

[TS 38.331, clause 5.3.3.7]

The UE shall:

1>	if timer T300 expires:

…

2>	store the following connection establishment failure information in the VarConnEstFailReport by setting its fields as follows:

…

3>	if available, set the locationInfo as follows:

4>	if available, set the commonLocationInfo to include the detailed location information;

[TS 38.331, clause 6.3.2]

The IE CommonLocationInfo is used to transfer detailed location information available at the UE to correlate measurements and UE position information.

CommonLocationInfo field descriptions

LocationTimeStamp Parameter type DisplacementTimeStamp defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

locationCoordinate Parameter type LocationCoordinates defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

locationError Parameter LocationError defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

locationSource Parameter LocationSource defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

velocityEstimate Parameter type Velocity defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

8.1.6.1.4.5.3	Test description

8.1.6.1.4.5.3.1	Pre-test conditions

System Simulator:

- NR Cell 1

UE:

- None.

Preamble:

-	The UE's positioning engine (e.g. standalone GNSS receiver) should be provided with any necessary stimulus to allow it to provide the position. This shall be done by use of the test function Update UE Location Information defined in TS 38.509 [6], if supported by the UE according to pc_UpdateUE_LocationInformation. Otherwise, or in addition any other suitable method may also be used.

- The UE is in state 1N-A as defined in TS 38.508-1 [4], subclause 4.4A.

8.1.6.1.4.5.3.2	Test procedure sequence

Table 8.1.6.1.4.5.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Void | - | - | - | - |
| 2 | The SS transmits a Paging message. | <-- | NR RRC: Paging | - | - |
| 3 | The UE transmits an RRCSetupRequest message. | --> | NR RRC: RRCSetupRequest | - | - |
| 4 | The SS waits for 2sec to ensure that the UE detects T300 expiry. | - | - | - | - |
| 5 | Void | - | - | - | - |
| 6-8 | Steps 1 to 3 of the generic radio bearer establishment procedure in TS 38.508-1 table 4.5.4.2-3. | - | - | - | - |
| 9 | The UE transmits an RRCSetupComplete message and a SERVICE REQUEST message. | --> | NR RRC: RRCSetupComplete | - | - |
| 10-13 | Steps 5 to 8 of the generic radio bearer establishment procedure in TS 38.508-1 table 4.5.4.2-3. | - | - | - | - |
| 14 | The SS sends a UEInformationRequest message to get connEstFailReportReq. | <-- | NR RRC: UEInformationRequest | - | - |
| 15 | Check: Does the UE send a UEInformationResponse message with connEstFailReport with the IE commonLocationInfo-r16 is present? | --> | NR RRC: UEInformationResponse | 1 | P |

8.1.6.1.4.5.3.3	Specific message contents

Table 8.1.6.1.4.5.3.3-1: RRCSetupComplete (step 9, Table 8.1.6.1.4.5.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ue-MeasurementsAvailable-r16 | UE-MeasurementsAvailable-r16 with condition CEF |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.4.5.3.3-2: Void

Table 8.1.6.1.4.5.3.3-3: UEInformationResponse (step 15, Table 8.1.6.1.4.5.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| connEstFailReport-r16 SEQUENCE { |  |  |  |
| measResultFailedCell-r16 SEQUENCE { |  |  |  |
| cgi-Info SEQUENCE { |  |  |  |
| plmn-Identity-r16 | Any allowed value |  |  |
| cellIdentity-r16 | Any allowed value |  |  |
| trackingAreaCode-r16 | Not checked |  |  |
| } |  |  |  |
| measResult-r16 SEQUENCE { |  |  |  |
| cellResults-r16 SEQUENCE { |  |  |  |
| resultsSSB-Cell-r16 SEQUENCE { |  |  |  |
| rsrp | Not checked |  |  |
| rsrq | Not checked |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| rsIndexResults-r16 SEQUENCE { |  |  |  |
| resultsSSB-Indexes-r16 SEQUENCE (SIZE (1..maxNrofIndexesToReport2)) OF ResultsPerSSB-Index | 1 entry |  |  |
| ssb-Index | Any allowed value |  |  |
| ssb-Results SEQUENCE { |  |  |  |
| rsrp | Not checked |  |  |
| rsrq | Not checked |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| locationInfo-r16 SEQUENCE { |  |  |  |
| commonLocationInfo-r16 | Any allowed value |  |  |
| bt-LocationInfo-r16 | Not present |  |  |
| wlan-LocationInfo-r16 | Not present |  |  |
| sensor-LocationInfo-r16 | Not present |  |  |
| } |  |  |  |
| measResultNeighCells-r16 SEQUENCE { |  |  |  |
| measResultNeighCellListNR | Not present |  |  |
| measResultNeighCellListEUTRA | Not present |  |  |
| } |  |  |  |
| numberOfConnFail-r16 | Any allowed value |  |  |
| perRAInfoList-r16 | Any allowed value |  |  |
| timeSinceFailure-r16 | Any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

###### 8.1.6.1.4.6	Connection Establishment Failure / Logging and reporting / Reporting of Intra-frequency measurements

8.1.6.1.4.6.1	Test Purpose (TP)

(1)

with { UE has connection establishment failure information available with the intra-frequency measurement result }

ensure that {

when { UE receives a UEInformationRequest message with connEstFailReportReq set to true }

then { UE sends a UEInformationResponse message containing the measurement result for intra-frequency neighbouring cell }

}

8.1.6.1.4.6.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clauses 5.3.3.7, 5.7.10.3 and 6.2.2. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if connEstFailReportReq is set to true and the UE has connection establishment failure or connection resume failure information in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

2>	set timeSinceFailure in VarConnEstFailReport to the time that elapsed since the last connection establishment failure or connection resume failure in NR;

2>	set the connEstFailReport in the UEInformationResponse message to the value of connEstFailReport in VarConnEstFailReport;

2>	discard the connEstFailReport from VarConnEstFailReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

[TS 38.331, clause 5.3.3.7]

The UE shall:

1>	if timer T300 expires:

…

2>	store the following connection establishment failure information in the VarConnEstFailReport by setting its fields as follows:

…

3>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements for at most the following number of neighbouring cells: 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies per RAT and according to the following:

4>	for each neighbour cell included, include the optional fields that are available;

NOTE 2:	The UE includes the latest results of the available measurements as used for cell reselection evaluation, which are performed in accordance with the performance requirements as specified in TS 38.133 [14].

[TS 38.331, clause 6.2.2]

–	UEInformationResponse

The UEInformationResponse message is used by the UE to transfer information requested by the network.

…

ConnEstFailReport field descriptions

measResultFailedCell This field refers to the last measurement results taken in the cell, where connection establishment failure or connection resume failure happened.

measResultNeighCells This field refers to the neighbour cell measurements when connection establishment failure or connection resume failure happened.

numberOfConnFail This field is used to indicate the latest number of consecutive failed RRCSetup or RRCResume procedures in the same cell independent of RRC state transition.

numberOfPreamblesSent This field is used to indicate the number of random access preambles that were transmitted.

timeSinceFailure This field is used to indicate the time that elapsed since the connection (establishment or resume) failure. Value in seconds. The maximum value 172800 means 172800s or longer.

8.1.6.1.4.6.3	Test description

8.1.6.1.4.6.3.1	Pre-test conditions

System Simulator:

- NR Cell 1 and Cell 2

UE:

- None.

Preamble:

- The UE is in state 1N-A as defined in TS 38.508-1 [4], subclause 4.4A, on cell 1.

- System information combination NR-2 as defined in TS 38.508-1 [4] clause 4.4.3.1.2.

8.1.6.1.4.6.3.2	Test procedure sequence

Table 8.1.6.1.4.6.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.6.1.4.6.3.2-1: Time instances of cell power level and parameter changes (FR1)

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -85 | ”Off” | Only NR Cell 1 is available. (NOTE 1). |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -85 | -91 | The power level values are assigned to satisfy RCell 1 > RCell 2. |
| NOTE 1: Power level “Off” is defined in TS 38.508-1 Table 6.2.2.1-3. |  |  |  |  |  |

Table 8.1.6.1.4.6.3.2-2: Time instances of cell power level and parameter changes (FR2)

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -82 | "Off" | Only NR Cell 1 is available. (NOTE 1). |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -91 | The power level values are assigned to satisfy RCell 1 > RCell 2. |
| NOTE 1: Power level “Off” is defined in TS 38.508-1 Table 6.2.2.2-3. |  |  |  |  |  |

Table 8.1.6.1.4.6.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes NR Cell 1 and NR Cell 2 parameter according to the row "T1" in Table 8.1.6.1.4.6.3.2-1/2. | - | - | - | - |
| 2 | The SS waits for 40s to ensure that the UE detects intra-frequency cell. | - | - | - | - |
| 3 | Void | - | - | - | - |
| 4 | The SS transmits a Paging message. | <-- | NR RRC: Paging | - | - |
| 5 | The UE transmits an RRCSetupRequest message. | --> | NR RRC: RRCSetupRequest | - | - |
| 6 | The SS waits for 2sec to ensure that the UE detects T300 expiry. | - | - | - | - |
| 7 | Void | - | - | - | - |
| 8-10 | Steps 1 to 3 of the generic radio bearer establishment procedure in TS 38.508-1 table 4.5.4.2-3 on NR Cell 1. | - | - | - | - |
| 11 | The UE transmits an RRCSetupComplete message and a SERVICE REQUEST message on NR Cell 1. | --> | NR RRC: RRCSetupComplete 5GMM: SERVICE REQUEST | - | - |
| 12-15 | Steps 5 to 8 of the generic radio bearer establishment procedure in TS 38.508-1 table 4.5.4.2-3 on NR Cell 1. | - | - | - | - |
| 16 | The SS sends a UEInformationRequest message to get connEstFailReportReq on NR Cell 1. | <-- | NR RRC: UEInformationRequest | - | - |
| 17 | Check: Does the UE send a UEInformationResponse message with connEstFailReport on NR Cell 1? | --> | NR RRC: UEInformationResponse | 1 | P |

8.1.6.1.4.6.3.3	Specific message contents

Table 8.1.6.1.4.6.3.3-1: RRCSetupComplete (step 11, Table 8.1.6.1.4.6.3.2-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ue-MeasurementsAvailable-r16 | UE-MeasurementsAvailable-r16 with condition CEF |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.4.6.3.3-2: Void

Table 8.1.6.1.4.6.3.3-3: UEInformationResponse (step 17, Table 8.1.6.1.4.6.3.2-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| connEstFailReport-r16 SEQUENCE { |  |  |  |
| measResultFailedCell-r16 SEQUENCE { |  |  |  |
| CGI-Info-Logging-r16 SEQUENCE { |  | Information of NR Cell 2 |  |
| plmn-Identity-r16 | the same MCC and MNC stored in EFIMSI on the test USIM card |  |  |
| cellIdentity-r16 | NR Cell 1 |  |  |
| trackingAreaCode-r16 | 1 |  |  |
| } |  |  |  |
| measResult-r16 SEQUENCE { |  |  |  |
| cellResults-r16 SEQUENCE { |  | Measurement result of NR Cell 2 |  |
| resultsSSB-Cell-r16 SEQUENCE { |  |  |  |
| rsrp | as specified in TS 38.508-1 Table 4.6.3-152 | Integer value for RSRP measurements |  |
| rsrq | as specified in TS 38.508-1 Table 4.6.3-153 | Integer value for RSRQ measurements |  |
| sinr | as specified in TS 38.508-1 Table 4.6.3-172 | Integer value for SINR measurements |  |
| } |  |  |  |
| } |  |  |  |
| rsIndexResults-r16 SEQUENCE { |  |  |  |
| resultsSSB-Indexes-r16 SEQUENCE (SIZE (1..maxNrofIndexesToReport2)) OF ResultsPerSSB-Index { | 1 entry |  |  |
| ResultsPerSSB-Index[1] SEQUENCE { |  | entry 1 |  |
| ssb-Index | SSB-Index | an SS-Block within an SS-Burst |  |
| ssb-Results SEQUENCE { |  | MeasQuantityResults |  |
| rsrp | as specified in TS 38.508-1 Table 4.6.3-152 | Integer value for RSRP measurements |  |
| rsrq | as specified in TS 38.508-1 Table 4.6.3-153 | Integer value for RSRQ measurements |  |
| sinr | as specified in TS 38.508-1 Table 4.6.3-172 | Integer value for SINR measurements |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| locationInfo-r16 | Not checked |  |  |
| measResultNeighCells-r16 SEQUENCE { |  |  |  |
| measResultNeighCellListNR SEQUENCE (SIZE (1..maxFreq)) OF MeasResult2NR-r16 { | 1 entry |  |  |
| MeasResult2NR-r16[1] SEQUENCE { |  | entry 1 |  |
| ssbFrequency-r16 | ARFCN-ValueNR |  |  |
| refFreqCSI-RS-r16 | ARFCN-ValueNR |  |  |
| measResultList-r16 SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultNR SEQUENCE { | 1 entry | the log measurement results taken in the PCell upon detecting radio link failure or the source PCell upon handover failure. |  |
| MeasResultNR[1] SEQUENCE { |  | entry 1 |  |
| physCellId | PCI of NR Cell 2 |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | Not checked |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| resultsCSI-RS-Cell | Not checked |  |  |
| } |  |  |  |
| rsIndexResults | Not checked |  |  |
| } |  |  |  |
| cgi-Info | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| numberOfConnFail-r16 | Integer between 1-8 |  |  |
| perRAInfoList-r16 | Not checked |  |  |
| timeSinceFailure-r16 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

###### 8.1.6.1.4.7	Connection Establishment Failure / Logging and reporting / Reporting of Inter-frequency measurements

8.1.6.1.4.7.1	Test Purpose (TP)

(1)

with { UE has connection establishment failure information available with the inter-frequency measurement result }

ensure that {

when { UE receives a UEInformationRequest message with connEstFailReportReq set to true }

then { UE sends a UEInformationResponse message containing the measurement result for inter-frequency neighbouring cell }

}

8.1.6.1.4.7.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clauses 5.3.3.7, 5.7.10.3 and 6.2.2. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if connEstFailReportReq is set to true and the UE has connection establishment failure or connection resume failure information in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

2>	set timeSinceFailure in VarConnEstFailReport to the time that elapsed since the last connection establishment failure or connection resume failure in NR;

2>	set the connEstFailReport in the UEInformationResponse message to the value of connEstFailReport in VarConnEstFailReport;

2>	discard the connEstFailReport from VarConnEstFailReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

[TS 38.331, clause 5.3.3.7]

The UE shall:

1>	if timer T300 expires:

…

2>	store the following connection establishment failure information in the VarConnEstFailReport by setting its fields as follows:

…

3>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements for at most the following number of neighbouring cells: 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies per RAT and according to the following:

4>	for each neighbour cell included, include the optional fields that are available;

NOTE 2:	The UE includes the latest results of the available measurements as used for cell reselection evaluation, which are performed in accordance with the performance requirements as specified in TS 38.133 [14].

[TS 38.331, clause 6.2.2]

–	UEInformationResponse

The UEInformationResponse message is used by the UE to transfer information requested by the network.

…

ConnEstFailReport field descriptions

measResultFailedCell This field refers to the last measurement results taken in the cell, where connection establishment failure or connection resume failure happened.

measResultNeighCells This field refers to the neighbour cell measurements when connection establishment failure or connection resume failure happened.

numberOfConnFail This field is used to indicate the latest number of consecutive failed RRCSetup or RRCResume procedures in the same cell independent of RRC state transition.

numberOfPreamblesSent This field is used to indicate the number of random access preambles that were transmitted.

timeSinceFailure This field is used to indicate the time that elapsed since the connection (establishment or resume) failure. Value in seconds. The maximum value 172800 means 172800s or longer.

8.1.6.1.4.7.3	Test description

8.1.6.1.4.7.3.1	Pre-test conditions

System Simulator:

- NR Cell 1 and NR Cell 3

- System information combination NR-4 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

- None.

Preamble:

- The UE is in state 1N-A as defined in TS 38.508-1 [4], subclause 4.4A, on NR Cell 1.

8.1.6.1.4.7.3.2	Test procedure sequence

Tables 8.1.6.1.4.7.3.2-1/2 illustrate the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.6.1.4.7.3.2-1: Time instances of cell power level and parameter changes (FR1)

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -85 | ”Off” | Only NR Cell 1 is available. (NOTE 1). |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -85 | -91 | The power level values are assigned to satisfy RNRCell 1 > RNRCell 3. |
| NOTE 1: Power level “Off” is defined in TS 38.508-1 [4], Table 6.2.2.1-3. |  |  |  |  |  |

Table 8.1.6.1.4.7.3.2-2: Time instances of cell power level and parameter changes (FR2)

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -82 | ”Off” | Only NR Cell 1 is available. (NOTE 1). |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -91 | The power level values are assigned to satisfy RNRCell 1 > RNRCell 3. |
| NOTE 1: Power level “Off” is defined in TS 38.508-1 [4], Table 6.2.2.2.2-2. |  |  |  |  |  |

Table 8.1.6.1.4.7.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes NR Cell 1 and NR Cell 3 parameter according to the row "T1" in Table 8.1.6.1.4.7.3.2-1/2. | - | - | - | - |
| 2 | The SS waits for 40s to ensure that the UE detects inter-frequency cell. | - | - | - | - |
| 3 | Void | - | - | - | - |
| 4 | The SS transmits a Paging message. | <-- | NR RRC: Paging | - | - |
| 5 | The UE transmits an RRCSetupRequest message. | --> | NR RRC: RRCSetupRequest | - | - |
| 6 | The SS waits for 2 sec to ensure that the UE detects T300 expiry. | - | - | - | - |
| 7 | Void | - | - | - | - |
| 8-10 | Steps 1 to 3 of the generic radio bearer establishment procedure in TS 38.508-1 [4], table 4.5.4.2-3 on NR Cell 1. | - | - | - | - |
| 11 | The UE transmits an RRCSetupComplete message and a SERVICE REQUEST message on NR Cell 1. | --> | NR RRC: RRCSetupComplete | - | - |
| 12-15 | Steps 5 to 8 of the generic radio bearer establishment procedure in TS 38.508-1 [4], table 4.5.4.2-3 on NR Cell 1. | - | - | - | - |
| 16 | The SS sends a UEInformationRequest message to get connEstFailReportReq. | <-- | NR RRC: UEInformationRequest | - | - |
| 17 | Check: Does the UE send a UEInformationResponse message with connEstFailReport on NR Cell 1? | --> | NR RRC: UEInformationResponse | 1 | P |

8.1.6.1.4.7.3.3	Specific message contents

Table 8.1.6.1.4.7.3.3-1: RRCSetupComplete (step 11, Table 8.1.6.1.4.7.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ue-MeasurementsAvailable-r16 | UE-MeasurementsAvailable-r16 with condition CEF |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.4.7.3.3-2: Void

Table 8.1.6.1.4.7.3.3-3: UEInformationResponse (step 17, Table 8.1.6.1.4.7.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| connEstFailReport-r16 SEQUENCE { |  |  |  |
| measResultFailedCell-r16 SEQUENCE { |  |  |  |
| cgi-Info SEQUENCE { |  | Information of NR Cell 1 |  |
| plmn-Identity-r16 | the same MCC and MNC stored in EFIMSI on the test USIM card |  |  |
| cellIdentity-r16 | NR Cell 1 |  |  |
| trackingAreaCode-r16 | 1 |  |  |
| } |  |  |  |
| measResult-r16 SEQUENCE { |  |  |  |
| cellResults-r16 SEQUENCE { |  | Measurement result of NR Cell 1 |  |
| resultsSSB-Cell-r16 SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| rsIndexResults-r16 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| locationInfo-r16 | Not checked |  |  |
| measResultNeighCells-r16 SEQUENCE { |  |  |  |
| measResultNeighCellListNR SEQUENCE (SIZE (1..maxFreq)) OF MeasResult2NR-r16 { | 1 entry |  |  |
| MeasResult2NR-r16[1] SEQUENCE { |  | entry 1 |  |
| ssbFrequency-r16 | ARFCN-ValueNR for SSB of NR Cell 3 |  |  |
| refFreqCSI-RS-r16 | Not checked |  |  |
| measResultList-r16 SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultNR SEQUENCE { | 1 entry |  |  |
| MeasResultNR[1] SEQUENCE { |  | entry 1 |  |
| physCellId | PCI of NR Cell 3 |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | Not checked |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| resultsCSI-RS-Cell | Not checked |  |  |
| } |  |  |  |
| rsIndexResults | Not checked |  |  |
| } |  |  |  |
| cgi-Info | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| numberOfConnFail-r16 | Integer between 1-8 |  |  |
| perRAInfoList-r16 | Not checked |  |  |
| timeSinceFailure-r16 | Any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

###### 8.1.6.1.4.8	Connection Establishment Failure / Logging and reporting / RACH failure report

8.1.6.1.4.8.1	Test Purpose (TP)

(1)

with { UE having previously failed the random access procedure where contention resolution was not successful for any of the transmitted preambles }

ensure that {

when { UE receives UEInformationRequest message with connEstFailReportReq set to true }

then { UE transmits UEInformationResponse message with a connEstFailReport including the perRAInfoList }

}

8.1.6.1.4.8.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clauses 5.3.3.7, 5.7.10.3, 5.7.10.4, 5.7.10.5 and 6.2.2. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.7.10.3]

1>	if timer T300 expires:

…

2>	store the following connection establishment failure information in the VarConnEstFailReport by setting its fields as follows:

…

3>	set perRAInfoList to indicate the performed random access procedure related information as specified in 5.7.10.5;

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if connEstFailReportReq is set to true and the UE has connection establishment failure or connection resume failure information in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

2>	set timeSinceFailure in VarConnEstFailReport to the time that elapsed since the last connection establishment failure or connection resume failure in NR;

2>	set the connEstFailReport in the UEInformationResponse message to the value of connEstFailReport in VarConnEstFailReport;

2>	discard the connEstFailReport from VarConnEstFailReport upon successful delivery of the UEInformationResponse message confirmed by lower layers; [TS 38.331, clause 5.7.10.5]

The UE shall set the content in ra-InformationCommon as follows:

…

1>	set the parameters associated to individual random-access attempt in the chronological order of attempts in the perRAInfoList as follows:

2>	if the random-access resource used is associated to a SS/PBCH block, set the associated random-access parameters for the successive random-access attempts associated to the same SS/PBCH block for one or more random-access attempts as follows:

3>	set the ssb-Index to include the SS/PBCH block index associated to the used random-access resource;

3>	set the numberOfPreamblesSentOnSSB to indicate the number of successive random-access attempts associated to the SS/PBCH block;

3>	for each random-access attempt performed on the random-access resource, include the following parameters in the chronological order of the random-access attempt:

4>	if the random-access attempt is performed on the contention based random-access resource and if raPurpose is not equal to 'requestForOtherSI', include contentionDetected as follows:

5>	if contention resolution was not successful as specified in TS 38.321 [6] for the transmitted preamble:

6>	set the contentionDetected to true;

5>	else:

6>	set the contentionDetected to false;

4>	if the random-access attempt is performed on the contention based random-access resource; or

4>	if the random-access attempt is performed on the contention free random-access resource and if the random-access procedure was initiated due to the PDCCH ordering:

5>	if the SS/PBCH block RSRP of the SS/PBCH block corresponding to the random-access resource used in the random-access attempt is above rsrp-ThresholdSSB:

6>	set the dlRSRPAboveThreshold to true;

5>	else:

6>	set the dlRSRPAboveThreshold to false;

2>	else if the random-access resource used is associated to a CSI-RS, set the associated random-access parameters for the successive random-access attempts associated to the same CSI-RS for one or more random-access attempts as follows:

3>	set the csi-RS-Index to include the CSI-RS index associated to the used random-access resource;

3>	set the numberOfPreamblesSentOnCSI-RS to indicate the number of successive random-access attempts associated to the CSI-RS.

[TS 38.331, clause 6.2.2]

–	UEInformationResponse

The UEInformationResponse message is used by the UE to transfer information requested by the network.

…

UEInformationResponse-IEs field descriptions

logMeasReport This field is used to provide the measurement results stored by the UE associated to logged MDT.

measResultIdleEUTRA EUTRA measurement results performed during RRC_INACTIVE or RRC_IDLE.

measResultIdleNR NR measurement results performed during RRC_INACTIVE or RRC_IDLE.

ra-Report This field is used to provide the list of RA reports that is stored by the UE for the past up to maxRAReport-r16 number of successful random access procedures.

rlf-Report This field is used to indicate the RLF report related contents.

…

RA-Report field descriptions

absoluteFrequencyPointA This field indicates the absolute frequency position of the reference resource block (Common RB 0).

cellID This field indicates the CGI of the cell in which the associated random access procedure was performed.

contentionDetected This field is used to indicate that contention was detected for the transmitted preamble in the given random access attempt or not. This field is not included when the UE performs random access attempt is using contention free random-access resources or when the raPurpose is set to requestForOtherSI.

csi-RS-Index This field is used to indicate the CSI-RS index corresponding to the random access attempt.

dlRSRPAboveThreshold This field is used to indicate whether the DL beam (SSB) quality associated to the random access attempt was above or below the threshold (rsrp-ThresholdSSB in beamFailureRecoveryConfig in UL BWP configuration of UL BWP selected for random access procedure initiated for beam failure recovery; Otherwise, rsrp-ThresholdSSB in rach-ConfigCommon in UL BWP configuration of UL BWP selected for random access procedure.

locationAndBandwidth Frequency domain location and bandwidth of the bandwidth part associated to the random-access resources used by the UE.

numberOfPreamblesSentOnCSI-RS This field is used to indicate the total number of successive RA preambles that were transmitted on the corresponding CSI-RS.

numberOfPreamblesSentOnSSB This field is used to indicate the total number of successive RA preambles that were transmitted on the corresponding SS/PBCH block.

perRAAttemptInfoList This field provides detailed information about a random access attempt.

perRAInfoList This field provides detailed information about each of the random access attempts in the chronological order of the random access attempts.

perRACSI-RSInfoList This field provides detailed information about the successive random access attempts associated to the same CSI-RS.

perRASSBInfoList This field provides detailed information about the successive random access attempts associated to the same SS/PBCH block.

raPurpose This field is used to indicate the RA scenario for which the RA report entry is triggered. The RA accesses associated to Initial access from RRC_IDLE, transition from RRC-INACTIVE and the MSG3 based SI request are indicated using the indicator 'accessRelated'. The indicator beamFailureRecovery is used in case of beam failure recovery failure in the SpCell [3]. The indicator reconfigurationWithSync is used if the UE executes a reconfiguration with sync. The indicator ulUnSynchronized is used if the random access procedure is initiated in a SpCell by DL or UL data arrival during RRC_CONNECTED when the timeAlignmentTimer is not running in the PTAG or in a serving cell by a PDCCH order [3]. The indicator schedulingRequestFailure is used in case of SR failures [3]. The indicator noPUCCHResourceAvailable is used when the UE has no valid SR PUCCH resources configured [3]. The indicator requestForOtherSI is used for MSG1 based on demand SI request.

ra-InformationCommon This field is used to indicate the common random-access related information between RA-report and RLF-report. For RA report, this field is mandatory presented. For RLF-report, this field is optionally included when connectionFailureType is set to 'hof' or when connectionFailureType is set to 'rlf' and the rlf-Cause equals to 'randomAccessProblem' or 'beamRecoveryFailure'; otherwise this field is absent.

ssb-Index This field is used to indicate the SS/PBCH index of the SS/PBCH block corresponding to the random access attempt.

subcarrierSpacing Subcarrier spacing used in the BWP associated to the random-access resources used by the UE.

…

8.1.6.1.4.8.3	Test description

8.1.6.1.4.8.3.1	Pre-test conditions

System Simulator:

- NR Cell 1

- System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.2.

UE:

- None.

Preamble:

- The UE is in state 1N-A on Cell 1 as defined in TS 38.508-1 [4], subclause 4.4A.

8.1.6.1.4.8.3.2	Test procedure sequence

Table 8.1.6.1.4.8.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS is configured to not transmit MAC control element and ignore subsequent RACH attempts | - | - | - | - |
| 1A | The SS transmits a Paging message. | <-- | NR RRC: Paging | - | - |
| 1B | The UE transmits an RRCSetupRequest message. | --> | NR RRC: RRCSetupRequest | - | - |
| 1C | The SS waits for 2sec to ensure that the UE detects T300 expiry. | - | - | - | - |
| 1D | SS is configured to transmit MAC control element with matched UE Contention Resolution Identity | - | - | - | - |
| 2 | SS sends a Paging message to the UE on the appropriate paging block, and including the UE identity in one entry of the IE pagingRecordList. | <-- | NR RRC: Paging | - | - |
| 3-9 | Steps 2-8 of generic test procedure in TS 38.508-1 Table 4.5.4.2-3 are performed on NR Cell 1. | - | - | - | - |
| 10 | The SS sends a UEInformationRequest message to get connEstFailReportReq. | <-- | NR RRC: UEInformationRequest | - | - |
| 11 | Check: Does the UE send a UEInformationResponse message with connEstFailReport including the perRAInfoList on NR Cell 1? | --> | NR RRC: UEInformationResponse | 1 | P |

8.1.6.1.4.8.3.3	Specific message contents

Table 8.1.6.1.4.8.3.3-1: Void

Table 8.1.6.1.4.8.3.3-2: UEInformationResponse (step 11, Table 8.1.6.1.4.8.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| connEstFailReport-r16 SEQUENCE { |  |  |  |
| measResultFailedCell-r16 SEQUENCE { |  |  |  |
| cgi-Info SEQUENCE { |  |  |  |
| plmn-Identity-r16 | plmn-Identity within SIB1 broadcasted in NR Cell 1 |  |  |
| cellIdentity-r16 | cellIdentity within SIB1 broadcasted in NR Cell 1 | BIT STRING (SIZE (36)) |  |
| trackingAreaCode-r16 | Not checked | BIT STRING (SIZE (24)) |  |
| } |  |  |  |
| measResult-r16 SEQUENCE { |  |  |  |
| cellResults-r16 | Any allowed value |  |  |
| rsIndexResults-r16 | Any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| locationInfo-r16 | Not checked |  |  |
| measResultNeighCells-r16 SEQUENCE { |  |  |  |
| measResultNeighCellListNR | Not present |  |  |
| measResultNeighCellListEUTRA | Not present |  |  |
| } |  |  |  |
| numberOfConnFail-r16 | Any allowed value | INTEGER (1..8) |  |
| perRAInfoList-r16 SEQUENCE (SIZE (1..200)) OF PerRAInfo-r16{ | 1 entry |  |  |
| PerRAInfo-r16[1] CHOICE { |  | entry 1 |  |
| perRASSBInfoList-r16 SEQUENCE { |  |  |  |
| ssb-Index-r16 | 1 |  |  |
| numberOfPreamblesSentOnSSB-r16 | Any allowed value | total number of successive RA preambles sent by the UE before T300 expiry |  |
| perRAAttemptInfoList-r16 SEQUENCE (SIZE (1..200)) OF PerRAAttemptInfo-r16{ |  |  |  |
| PerRAAttemptInfo-r16[x] SEQUENCE { |  | x ranges from 1 to numberOfPreamblesSentOnSSB-r16 |  |
| contentionDetected-r16 | true | entry 1 |  |
|  | false | entry 2 to x |  |
| dlRSRPAboveThreshold-r16 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| timeSinceFailure-r16 | Any allowed value | INTEGER (0..172800) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

###### 8.1.6.1.4.9	Connection Establishment Failure / Logging and reporting / T300 expiry / Multiple CEF reports

8.1.6.1.4.9.1	Test Purpose (TP)

(1)

with { UE having sent an RRCSetupComplete message with connEstFailInfoAvailable }

ensure that {

when { T300 is expired and random access procedure failed on the current cell with cell identity not equal to the cell identity stored in measResultFailedCell and UE receives a UEInformationRequest message with connEstFailReportReq set to true }

then { UE sends a UEInformationResponse message with ConnEstFailReportList-r17 }

}

(2)

with { UE having sent an RRCSetupComplete message with connEstFailInfoAvailable }

ensure that {

when { T300 is expired and random access procedure failed twice on the same cell with cell identity of current cell is equal to the cell identity stored in measResultFailedCell and UE receives a UEInformationRequest message with connEstFailReportReq set to true }

then { UE sends a UEInformationResponse message with ConnEstFailReportList-r17 and does not append the existing VarConnEstFailReportList }

}

8.1.6.1.4.9.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clauses 5.3.3.4, 5.3.3.7 and 5.7.10.3. Unless otherwise stated these are Rel-17 requirements.

[TS 38.331, clause 5.3.3.4]

The UE shall perform the following actions upon reception of the RRCSetup:

…

1>	set the content of RRCSetupComplete message as follows:

…

2>	if the UE has connection establishment failure or connection resume failure information available in VarConnEstFailReport or VarConnEstFailReportList and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport or in at least one of the entries of VarConnEstFailReportList:

3>	include connEstFailInfoAvailable in the RRCSetupComplete message;

…

1>	submit the RRCSetupComplete message to lower layers for transmission, upon which the procedure ends.

[TS 38.331, clause 5.3.3.7]

The UE shall:

1>	if timer T300 expires:

2>	reset MAC, release the MAC configuration and re-establish RLC for all RBs that are established (except broadcast MRBs);

2>	if the UE supports RRC Connection Establishment failure with temporary offset and the T300 has expired a consecutive connEstFailCount times on the same cell for which connEstFailureControl is included in SIB1:

3>	for a period as indicated by connEstFailOffsetValidity:

4>	use connEstFailOffset for the parameter Qoffsettemp for the concerned cell when performing cell selection and reselection according to TS 38.304 [20] and TS 36.304 [27];

NOTE 1:	When performing cell selection, if no suitable or acceptable cell can be found, it is up to UE implementation whether to stop using connEstFailOffset for the parameter Qoffsettemp during connEstFailOffsetValidity for the concerned cell.

2>	if the UE supports multiple CEF report:

3>	if the UE has connection establishment failure information or connection resume failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-identity stored in VarConnEstFailReport; and

3>	if the cell identity of current cell is not equal to the cell identity stored in measResultFailedCell in VarConnEstFailReport and if the maxCEFReport-r17 has not been reached:

4>	append the VarConnEstFailReport as a new entry in the VarConnEstFailReportList;

2>	if the UE has connection establishment failure information or connection resume failure information available in VarConnEstFailReport and if the RPLMN is not equal to plmn-identity stored in VarConnEstFailReport; or

2>	if the cell identity of current cell is not equal to the cell identity stored in measResultFailedCell in VarConnEstFailReport:

3>	reset the numberOfConnFail to 0;

2>	if the UE supports multiple CEF report and if the UE has connection establishment failure informatoin or connection resume failure information available in VarConnEstFailReportList and if the RPLMN is not equal to plmn-identity stored in any entry of VarConnEstFailReportList:

3>	clear the content included in VarConnEstFailReportList;

2>	clear the content included in VarConnEstFailReport except for the numberOfConnFail, if any;

2>	store the following connection establishment failure information in the VarConnEstFailReport by setting its fields as follows:

3>	set the plmn-Identity to the PLMN selected by upper layers (see TS 24.501 [23]) from the PLMN(s) included in the plmn-IdentityInfoList in SIB1;

3>	set the measResultFailedCell to include the global cell identity, tracking area code, the cell level and SS/PBCH block level RSRP, and RSRQ, and SS/PBCH block indexes, of the failed cell based on the available SSB measurements collected up to the moment the UE detected connection establishment failure;

3>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements for at most the following number of neighbouring cells: 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies per RAT and according to the following:

4>	for each neighbour cell included, include the optional fields that are available;

NOTE 2:	The UE includes the latest results of the available measurements as used for cell reselection evaluation, which are performed in accordance with the performance requirements as specified in TS 38.133 [14].

3>	if available, set the locationInfo as follows:

4>	if available, set the commonLocationInfo to include the detailed location information;

4>	if available, set the bt-LocationInfo to include the Bluetooth measurement results, in order of decreasing RSSI for Bluetooth beacons;

4>	if available, set the wlan-LocationInfo to include the WLAN measurement results, in order of decreasing RSSI for WLAN APs;

4>	if available, set the sensor-LocationInfo to include the sensor measurement results as follows;

5>	if available, include the sensor-MeasurementInformation;

5>	if available, include the sensor-MotionInformation;

NOTE 3:	Which location information related configuration is used by the UE to make the locationInfo available for inclusion in the VarConnEstFailReport is left to UE implementation.

3>	set perRAInfoList to indicate the performed random access procedure related information as specified in 5.7.10.5;

3>	if the numberOfConnFail is smaller than 8:

4>	increment the numberOfConnFail by 1;

2>	inform upper layers about the failure to establish the RRC connection, upon which the procedure ends;

The UE may discard the connection establishment failure or connection resume failure information, i.e. release the UE variable VarConnEstFailReport and the UE variable VarConnEstFailReportList, 48 hours after the last connection establishment failure is detected.

The L2 U2N Relay UE either indicates to upper layers (to trigger PC5 unicast link release) or sends Notification message to the connected L2 U2N Remote UE(s) in accordance with 5.8.9.10.

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if connEstFailReportReq is set to true and the UE has connection establishment failure or connection resume failure information in VarConnEstFailReport or VarConnEstFailReportList and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport or in at least one of the entries of VarConnEstFailReportList:

2>	set timeSinceFailure in VarConnEstFailReport to the time that elapsed since the last connection establishment failure or connection resume failure in NR;

2>	set the connEstFailReport in the UEInformationResponse message to the value of connEstFailReport in VarConnEstFailReport;

2>	if the UE supports multiple CEF report:

3>	for each connEstFailReport in the connEstFailReportList in VarConnEstFailReportList:

4>	set timeSinceFailure to the time that elapsed since the associated connection establishment failure or connection resume failure in NR;

2>	for each connEstFailReport in the connEstFailReportList in the UEInformationResponse message, set the value to the value of connEstFailReport in VarConnEstFailReport in VarConnEstFailReportList;

2>	discard the connEstFailReport from VarConnEstFailReport and VarConnEstFailReportList upon successful delivery of the UEInformationResponse message confirmed by lower layers;

…

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1.

8.1.6.1.4.9.3	Test description

8.1.6.1.4.9.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 and NR Cell 3.

-	System information combination NR-4 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

-	None.

Preamble:

-	The UE is in state 1N-A as defined in TS 38.508-1 [4], subclause 4.4A, on NR Cell 1.

8.1.6.1.4.9.3.2	Test procedure sequence

Table 8.1.6.1.4.9.3.2-1/2 illustrates the downlink power levels and other changing parameters to be applied for the NR cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. The configuration "T0" indicates the initial conditions. Subsequent configurations marked "T1" are applied at the points indicated in the Main behaviour description in Table 8.1.6.1.4.9.3.2-3.

Table 8.1.6.1.4.9.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | Off | The power level values are assigned to ensure the UE registered on NR Cell 1. |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -78 | The power level values are assigned to satisfy RNR Cell 1 < RNR Cell 3. |

Table 8.1.6.1.4.9.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -91 | Off | The power level values are assigned to ensure the UE registered on NR Cell 1. |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -91 | -82 | The power level values are assigned to satisfy RNR Cell 1 < RNR Cell 3. |

Table 8.1.6.1.4.9.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a Paging message. | <-- | NR RRC: Paging | - | - |
| 2 | The UE transmits an RRCSetupRequest message. | --> | NR RRC: RRCSetupRequest | - | - |
| 3 | The SS waits for 2sec to ensure that the UE detects T300 expiry. | - | - | - | - |
| 4 | The SS transmits a Paging message. | <-- | NR RRC: Paging | - | - |
| 5-11 | Steps 2 to 8 of procedure described in TS 38.508-1 [4] table 4.5.4.2-3: NR RRC_CONNECTED are performed. | - | - | - | - |
| 12 | The SS transmits RRCRelease message on NR Cell 1. | <-- | NR RRC: RRCRelease | - | - |
| 13 | The SS changes NR Cell 3 SS PBCH SSS EPRE level according to the row "T1" in table 8.1.6.1.4.9.3.2-1/2. | - | - | - | - |
| 14 | Wait for 66[FR1]/258[FR2] seconds to allow UE to recognise the change. | - | - | - | - |
| - | The following messages are to be observed on NR Cell 3 unless explicitly stated otherwise. | - | - | - | - |
| 15 | The SS transmits a Paging message. | <-- | NR RRC: Paging | - | - |
| 16 | The UE transmits an RRCSetupRequest message. | --> | NR RRC: RRCSetupRequest | - | - |
| 17 | The SS waits for 2sec to ensure that the UE detects T300 expiry. | - | - | - | - |
| 18 | The SS transmits a Paging message. | <-- | NR RRC: Paging | - | - |
| 19 | The UE transmits an RRCSetupRequest message. | --> | NR RRC: RRCSetupRequest | - | - |
| 20 | The SS transmits an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 21 | Check: Does the UE transmit an RRCSetupComplete message including connEstFailInfoAvailable IE set it to true? UE initiates the session management procedure by including the SERVICE REQUEST message. | --> | NR RRC: RRCSetupComplete 5GMM: SERVICE REQUEST | 1 | P |
| 22-25 | Steps 5 to 8 of the generic radio bearer establishment procedure in TS 38.508-1 [4] table 4.5.4.2-3. | - | - | - | - |
| 26-27 | Void | - | - | - | - |
| 28 | The SS transmits an RRCRelease message | <-- | NR RRC: RRCRelease | - | - |
| 29 | The SS transmits a Paging message. | <-- | NR RRC: Paging | - | - |
| 30 | The UE transmits an RRCSetupRequest message. | --> | NR RRC: RRCSetupRequest | - | - |
| 31 | The SS waits for 2sec to ensure that the UE detects T300 expiry. | - | - | - | - |
| 32 | The SS transmits a Paging message. | <-- | NR RRC: Paging | - | - |
| 33 | The UE transmits an RRCSetupRequest message. | --> | NR RRC: RRCSetupRequest | - | - |
| 34 | The SS transmits an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 35 | Check: Does the UE transmit an RRCSetupComplete message including connEstFailInfoAvailable IE set it to true? UE initiates the session management procedure by including the SERVICE REQUEST message. | --> | NR RRC: RRCSetupComplete 5GMM: SERVICE REQUEST | 2 | P |
| 36-39 | Steps 5 to 8 of the generic radio bearer establishment procedure in TS 38.508-1 [4] table 4.5.4.2-3. | - | - | - | - |
| 40 | The SS sends a UEInformationRequest message to get connEstFailReportReq. | <-- | NR RRC: UEInformationRequest | - | - |
| 41 | Check: Does the UE send a UEInformationResponse message with ConnEstFailReportList-r17? | --> | NR RRC: UEInformationResponse | 1,2 | P |

8.1.6.1.4.9.3.3	Specific message contents

Table 8.1.6.1.4.9.3.3-1: RRCSetupComplete (steps 7, 21 and 35, Table 8.1.6.1.4.9.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ue-MeasurementsAvailable-r16 | UE-MeasurementsAvailable-r16 with condition CEF |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.4.9.3.3-2: UEInformationResponse (step 41, Table 8.1.6.1.4.9.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| connEstFailReportList-r17 SEQUENCE (SIZE (1..maxCEFReport-r17)) OF ConnEstFailReport-r16 { | 2 entries |  |  |
| ConnEstFailReport-r16[1] SEQUENCE { |  | entry 1 |  |
| measResultFailedCell-r16 SEQUENCE { |  |  |  |
| cgi-Info SEQUENCE { |  |  |  |
| plmn-Identity-r16 | plmn-Identity within SIB1 broadcasted in NR Cell 1 |  |  |
| cellIdentity-r16 | cellIdentity within SIB1 broadcasted in NR Cell 1 | BIT STRING (SIZE (36)) |  |
| trackingAreaCode-r16 | Not present or any allowed value | BIT STRING (SIZE (24)) |  |
| } |  |  |  |
| measResult-r16 SEQUENCE { |  |  |  |
| cellResults-r16 | Not present or any allowed value |  |  |
| rsIndexResults-r16 | Not present or any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| locationInfo-r16 | Not present or any allowed value |  |  |
| measResultNeighCells-r16 SEQUENCE { |  |  |  |
| measResultNeighCellListNR | Not present |  |  |
| measResultNeighCellListEUTRA | Not present |  |  |
| } |  |  |  |
| numberOfConnFail-r16 | Any allowed value | INTEGER (1..8) |  |
| perRAInfoList-r16 | Any allowed value |  |  |
| timeSinceFailure-r16 | Any allowed value | INTEGER (0..172800) |  |
| } |  |  |  |
| ConnEstFailReport-r16[2] SEQUENCE { |  | entry 2 |  |
| measResultFailedCell-r16 SEQUENCE { |  |  |  |
| cgi-Info SEQUENCE { |  |  |  |
| plmn-Identity-r16 | plmn-Identity within SIB1 broadcasted in NR Cell 3 |  |  |
| cellIdentity-r16 | cellIdentity within SIB1 broadcasted in NR Cell 3 | BIT STRING (SIZE (36)) |  |
| trackingAreaCode-r16 | Not present or any allowed value | BIT STRING (SIZE (24)) |  |
| } |  |  |  |
| measResult-r16 SEQUENCE { |  |  |  |
| cellResults-r16 | Not present or any allowed value |  |  |
| rsIndexResults-r16 | Not present or any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| locationInfo-r16 | Not present or any allowed value |  |  |
| measResultNeighCells-r16 SEQUENCE { |  |  |  |
| measResultNeighCellListNR | Not present |  |  |
| measResultNeighCellListEUTRA | Not present |  |  |
| } |  |  |  |
| numberOfConnFail-r16 | Any allowed value | INTEGER (1..8) |  |
| perRAInfoList-r16 | Any allowed value |  |  |
| timeSinceFailure-r16 | Any allowed value | INTEGER (0..172800) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.4.9.3.3-3: Void