3GPP TS 36.424 V15.1.0 (2019-12)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Evolved Universal Terrestrial Radio Access Network
(E-UTRAN);

X2 data transport

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

Transport, Tunnelling, LTE, radio

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definitions, symbols and abbreviations	5

3.1	Definitions	5

3.2	Abbreviations	6

4	Data link layer	6

5	X2 interface user plane protocol	6

5.1	General	6

5.2	GTP-U	7

5.3	UDP/IP	7

5.4	Diffserv code point marking	7

5.5	Dual Connectivity	7

5.6	E-UTRA-NR Dual Connectivity	8

Annex A (informative): Change history	9

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document specifies the standards for user data transport protocols and related signalling protocols to establish user plane transport bearers over the X2 interface.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 29.281: "General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)".

[3]	IETF RFC 768 (1980-08): "User Datagram Protocol".

[4]	IETF RFC 2474 (1998-12): "Definition of the Differentiated Services Field (DS Field) in the Ipv4 and Ipv6 Headers".

[5]	IETF RFC 2460 (1998-12): "Internet Protocol, Version 6 (IPv6) Specification".

[6]	IETF RFC 791 (1981-09): "Internet Protocol".

[7]	3GPP TS 36.401: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Architecture Description".

[8]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[9]	3GPP TS 36.425: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 interface user plane protocol".

[10]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage-2".

[11]	3GPP TS 38.425; "NG-RAN; NR user plane protocol".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the following terms and definitions below apply. Terms and definitions not defined below can be found in TR 21.905 [1].

Corresponding E-UTRAN node: Used in this specification according to the definition of the corresponding node in TS 38.425 [11].

Dual Connectivity: Defined in TS 36.300 [8].

EN-DC: Defined in TS 37.340 [10].

E-RAB: Defined in TS 36.401 [7].

X2: logical interface between two eNBs. Whilst logically representing a point to point link between eNBs, the physical realisation need not be a point to point link.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

DC	Dual Connectivity

eNB	E-UTRAN Node B

EN-DC	E-UTRA-NR Dual Connectivity

E-RAB	E-UTRAN Radio Access Bearer

E-UTRAN	Evolved UTRAN

GTP	GPRS Tunnelling Protocol

IP	Internet Protocol

MeNB	Master eNB

PDCP	Packet Data Convergence Protocol

PDU	Protocol Data Unit

SCG	Secondary Cell Group

SeNB	Secondary eNB

TEID	Tunnel Endpoint Identifier

UDP	User Datagram Protocol

# 4	Data link layer

Any data link protocol that fulfils the requirements toward the upper layer may be used.

# 5	X2 interface user plane protocol

## 5.1	General

The transport layer for data streams over X2 is an IP based Transport. The following figure shows the transport protocol stacks over X2.

Figure 5.1: Transport network layer for data streams over X2

The GTP-U (TS 29.281 [2]) protocol over UDP over IP shall be supported as the transport for data streams on the X2 interface. The data link layer is as specified in clause 4.

There may be zero or one UL data stream and zero or one DL data stream per E-RAB at the X2 interface.

-	The DL data stream is used for DL data forwarding from the source eNB to the target eNB.

-	The UL data stream is used for UL data forwarding from the source eNB to the target eNB.

Each data stream is carried on a dedicated transport bearer.

The identity of a transport bearer signalled in the RNL control plane consists of the IP address and the TEID of the corresponding GTP tunnel, allocated by the target eNB (see TS 29.281 [2]).

## 5.2	GTP-U

The GTP-U (TS 29.281 [2]) protocol shall be used over the X2 interface between two eNBs.

## 5.3	UDP/IP

The path protocol used shall be UDP (IETF RFC 768 [3]).

The UDP port number for GTP-U shall be as defined in TS 29.281 [2].

The eNBs over the X2 interface shall support fragmentation and assembly of GTP packets at the IP layer.

The eNB shall support IPv6 (IETF RFC 2460 [5]) and/or IPv4 (IETF RFC 791 [6]).

There may be one or several IP addresses in the both eNBs. The packet processing function in the source eNB shall send downstream packets of a given E-RAB to the target eNB IP address (received in X2AP) associated to the DL transport bearer of that particular E-RAB. The packet processing function in the source eNB shall send upstream packets of a given E-RAB to the target eNB IP address (received in X2AP) associated to the UL transport bearer of that particular E-RAB.

The Transport Layer Address signalled in X2AP messages is a bit string of

a)	32 bits in case of IPv4 address according to IETF RFC 791 [6]; or

b)	128 bits in case of IPv6 address according to IETF RFC 2460 [5] ; or

c)	160 bits if both IPv4 and IPv6 addresses are signalled, in which case the IPv4 address is contained in the first 32 bits.

## 5.4	Diffserv code point marking

IP Differentiated Services code point marking (IETF RFC 2474 [4]) shall be supported. The mapping between traffic categories and Diffserv code points shall be configurable by O&M based on QoS Class Identifier (QCI)/ Label Characteristics and other E-UTRAN traffic parameters (e.g. ARP). Traffic categories are implementation-specific and may be determined from the application parameters.

## 5.5	Dual Connectivity

For the SCG bearer option, user data forwarding may be performed. The behaviour of the eNB from which user data is forwarded is the same as specified for the “source eNB”, the behaviour of the eNB to which user data is forwarded is the same as specified for the “target eNB”.

For the split bearer option:

-	the GTP-U (TS 29.281 [2]) protocol over UDP over IP shall be supported as the transport for the data stream of PDCP PDUs on the X2 interface. The GTP-U PDU may include a RAN Container with flow control information as specified in TS 36.425 [9] which is carried in the GTP-U extension header. The transport bearer is identified by the GTP-U TEID (TS 29.281 [2]) and the IP address of the MeNB and SeNB respectively. There may be zero or one UL data stream and there is one DL data stream per E-RAB at the X2 interface;

-	The DL data stream is used for DL data transmission from the MeNB to the SeNB;

-	The UL data stream is used for UL data transmission from the SeNB to the MeNB;

-	the packet processing function in the MeNB shall send downstream packets of a given E-RAB to the SeNB IP address (received in X2AP) associated to the DL transport bearer of that particular E-RAB. The packet processing function in the SeNB shall send upstream packets of a given E-RAB to the MeNB IP address (received in X2AP) associated to the UL transport bearer of that particular E-RAB;

-	data forwarding may be performed by MeNB providing GTP-U TEID to receive the DL data forwarded by the SeNB.

## 5.6	E-UTRA-NR Dual Connectivity

User data forwarding may be performed for each E-RAB configured for EN-DC, towards or from the node hosting the PDCP entity. The behaviour of the E-UTRAN node from which user data is forwarded is the same as specified for the "source eNB", the behaviour of the E-UTRAN node to which user data is forwarded is the same as specified for the "target eNB".

If X2-U data bearer resources are allocated for EN-DC:

-	the GTP-U (TS 29.281 [2]) protocol over UDP over IP shall be supported as the transport for the data stream of PDCP PDUs on the X2 interface. The GTP-U PDU may include an NR RAN Container with flow control information as specified in TS 38.425 [11] which is carried in the GTP-U extension header. The transport bearer is identified by the GTP-U TEID (TS 29.281 [2]) and the IP address of the E-UTRAN nodes involved in EN-DC;

-	the packet processing function in the E-UTRAN node hosting the PDCP entity shall send downstream packets of a given E-RAB to the IP address indicated by the corresponding E-UTRAN node in X2AP associated to the DL transport bearer of that particular E-RAB. The packet processing function in the corresponding E-UTRAN node shall send upstream packets of a given E-RAB to the IP address indicated by the E-UTRAN node hosting the PDCP entity in X2AP associated to the UL transport bearer of that particular E-RAB;

Annex A (informative):
Change history

| TSG # | TSG Doc. | CR | Rev | Subject/Comment | New |
| --- | --- | --- | --- | --- | --- |
| 38 |  |  |  | approved at TSG-RAN and placed under change control | 8.0.0 |
| 39 | RP-080078 | 0001 | - | Editorial correction on 36.424 | 8.1.0 |
| 39 | RP-080078 | 0002 | - | Data link layer proposal | 8.1.0 |
| 40 | RP-080302 | 0003 | 1 | eGTP draft reference for X2 Data Transport | 8.2.0 |
| 40 | RP-080302 | 0005 | - | Define format for TLA signalled in X2AP messages | 8.2.0 |
| 41 | RP-080583 | 0006 | 1 | X2 transport bearers | 8.3.0 |
| 42 | RP-080845 | 0007 |  | Correction of SAE Bearer | 8.4.0 |
| 43 | RP-090083 | 0008 |  | Correction on GTP-U version | 8.5.0 |
| 09/2009 | - | - | - | Creation of Rel-9 version based on v8.5.0 | 9.0.0 |
| 12/2010 |  |  |  | Creation of Rel-10 version based on v. 9.0.0 | 10.0.0 |
| SP-49 | SP-100629 |  |  | Clarification on the use of References (TS 21.801 CR#0030) | 10.0.1 |
| 52 | RP-110684 | 0009 |  | Correction of references | 10.1.0 |
| 09/2012 |  |  |  | Update to Rel-11 version (MCC) | 11.0.0 |
| 63 | RP-140297 | 0011 | 1 | The content of Transport Layer Address | 12.0.0 |
| 66 | RP-142089 | 0013 | 6 | Data Forwarding and Data transmission | 12.1.0 |
| 67 | RP-150351 | 0021 | 1 | Correction on Data Transmission over X2 | 12.2.0 |
| 12/2015 |  |  |  | Update to Rel-13 version (MCC) | 13.0.0 |
| 71 | RP-160449 | 0022 | 1 | Rapporteur editorial corrections | 13.1.0 |

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-03 | SA#75 |  |  |  |  | Promotion to Release 14 without technical change | 14.0.0 |
| 2017-06 | RP#76 | RP-171324 | 0024 | 2 | F | Derivation of Diffserv code point marking includes ARP | 14.1.0 |
| 2017-06 | RP#76 | RP-171324 | 0025 |  | F | Clarification of the use of the RAN Container | 14.1.0 |
| 2017-06 | RP#76 | RP-171324 | 0026 |  | F | Rapporteur editorial review | 14.1.0 |
| 2017-12 | RP-78 | RP-172672 | 0027 | 1 | B | Introduction of EN-DC | 15.0.0 |
| 2019-12 | RP-86 | RP-192915 | 0029 | 1 | F | Independent migration to IPv6 on S1-U for en-gNB’s | 15.1.0 |