| 3GPP TS 26.117 V19.1.0 (2025-12) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 5G Media Streaming (5GMS); Speech and audio profiles (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	9

3.1	Terms	9

3.2	Symbols	9

3.3	Abbreviations	9

4	Overview	10

5	Media Capabilities	10

5.1	Introduction	10

5.2	Decoding Capabilities	10

5.3	Encoding Capabilities	11

5.4	Definition of simultaneous encoding and decoding capabilities	12

6	Operation Points	12

6.1	Introduction	12

6.2	Speech Operation Points	12

6.2.1	Introduction	12

6.2.2	AMR	13

6.2.2.1	Bitstream Encoding Requirements	13

6.2.2.2	Receiver Requirements	13

6.2.2.3	Sender Requirements	13

6.2.3	AMR-WB	13

6.2.3.1	Bitstream Requirements	13

6.2.3.2	Receiver Requirements	13

6.2.3.3	Sender Requirements	13

6.2.4	EVS	13

6.2.4.1	Bitstream Encoding Requirements	13

6.2.4.2	Receiver Requirements	14

6.2.4.3	Sender Requirements	14

6.3	Audio Operation Points	14

6.3.1	Introduction	14

6.3.2	eAAC+ stereo	14

6.3.2.1	Bitstream Encoding Requirements	14

6.3.2.2	Receiver Requirements	14

6.3.2.3	Sender Requirements	14

6.3.3	AMR-WB+	14

6.3.3.1	Bitstream Encoding Requirements	14

6.3.3.2	Receiver Requirements	15

6.3.3.3	Sender Requirements	15

6.3.4	xHE-AAC stereo	15

6.3.4.1	Bitstream Encoding Requirements	15

6.3.4.2	Receiver Requirements	15

6.3.4.3	Sender Requirements	15

6.3.5	IVAS	15

6.3.5.1	Bitstream Encoding Requirements	15

6.3.5.2	Receiver Requirements	16

6.3.5.3	Sender Requirements	16

6.3.6	AAC-ELDv2	16

6.3.6.1	Bitstream Encoding Requirements	16

6.3.6.2	Receiver Requirements	16

6.3.6.3	Sender Requirements	16

7	Mapping to 5GMS delivery	16

7.1	Introduction	16

7.2	AMR Media Profile	17

7.2.1	Mapping to ISO BMFF	17

7.2.2	Media Profile Definition	17

7.2.2.1	CMAF Track Definition	17

7.2.2.2	CMAF Switching Set and Media Profile Definition	17

7.2.2.3	Mapping to DASH Adaptation Set	17

7.2.2.4	Playback Requirements	17

7.2.2.5	Content Generation Requirements	18

7.3	AMR-WB Media Profile	18

7.3.1	Mapping to ISO BMFF	18

7.3.2	Media Profile Definition	18

7.3.2.1	CMAF Track Definition	18

7.3.2.2	CMAF Switching Set and Media Profile Definition	18

7.3.2.3	Mapping to DASH Adaptation Set	18

7.3.2.4	Playback Requirements	19

7.3.2.5	Content Generation Requirements	19

7.4	EVS Media Profile	19

7.4.1	Mapping to ISO BMFF	19

7.4.2	Media Profile Definition	20

7.4.2.1	CMAF Track Definition	20

7.4.2.2	CMAF Switching Set and Media Profile Definition	20

7.4.2.3	Mapping to DASH Adaptation Set	20

7.4.2.4	Playback Requirements	20

7.4.2.5	Content Generation Requirements	21

7.5	IVAS Media Profile	21

7.5.1	Mapping to ISO BMFF	21

7.5.2	Media Profile Definition	21

7.5.2.1	CMAF Track Definition	21

7.5.2.2	CMAF Switching Set and Media Profile Definition	21

7.5.2.3	Mapping to DASH Adaptation Set	21

7.5.2.4	Playback Requirements	22

7.5.2.5	Content Generation Requirements	22

7.6	eAAC+ stereo Media Profile	22

7.6.1	Void	22

7.6.2	Media Profile Definition	22

7.6.2.1	CMAF Track Definition	22

7.6.2.2	CMAF Switching Set and Media Profile Definition	23

7.6.2.3	Mapping to DASH Adaptation Set	23

7.6.2.4	Playback Requirements	23

7.6.2.5	Content Generation Requirements	24

7.7	AMR-WB+ Media Profiles	24

7.7.1	Mapping to ISO BMFF	24

7.7.2	Media Profile Definition	24

7.7.2.1	CMAF Track Definition	24

7.7.2.2	CMAF Switching Set and Media Profile Definition	24

7.7.2.3	Mapping to DASH Adaptation Set	24

7.7.2.4	Playback Requirements	24

7.7.2.5	Content Generation Requirements	25

7.8	xHE-AAC Media Profile	25

7.8.1	CMAF Track Definition	25

7.8.2	CMAF Switching Set and Media Profile Definition	25

7.8.3	Mapping to DASH Adaptation Set	26

7.8.4	Playback Requirements	26

7.8.5	Content Generation Requirements	26

Annex A (informative):	Registration Information	27

A.1	3GPP Registered URIs	27

Annex B (informative):	Change history	28

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies speech and audio media capabilities, operation points and media profiles for 5G Media Streaming in the context of 3GPP services and deployments. Speech and audio media capabilities, operation points and media profiles are also provided for usage in other streaming applications.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[3]	3GPP TS 26.071: "Mandatory Speech Codec speech processing functions; AMR Speech CODEC; General description".

[4]	3GPP TS 26.090: "Mandatory Speech Codec speech processing functions; Adaptive Multi-Rate (AMR) speech codec; Transcoding functions".

[5]	3GPP TS 26.073: "ANSI-C code for the Adaptive Multi Rate (AMR) speech codec".

[6]	3GPP TS 26.104: "ANSI-C code for the floating-point Adaptive Multi Rate (AMR) speech codec".

[7]	3GPP TS 26.093: "Mandatory speech codec speech processing functions; Adaptive Multi-Rate (AMR) speech codec; Source controlled rate operation".

[8]	3GPP TS 26.171: "Speech codec speech processing functions; Adaptive Multi-Rate - Wideband (AMR-WB) speech codec; General description".

[9]	3GPP TS 26.190: "Speech codec speech processing functions; Adaptive Multi-Rate - Wideband (AMR-WB) speech codec; Transcoding functions".

[10]	3GPP TS 26.173: "ANCI-C code for the Adaptive Multi Rate - Wideband (AMR-WB) speech codec".

[11]	3GPP TS 26.204: "Speech codec speech processing functions; Adaptive Multi-Rate - Wideband (AMR-WB) speech codec; ANSI-C code".

[12]	3GPP TS 26.193: "Speech codec speech processing functions; Adaptive Multi-Rate - Wideband (AMR-WB) speech codec; Source controlled rate operation".

[13]	3GPP TS 26.441: "Codec for Enhanced Voice Services (EVS); General Overview".

[14]	3GPP TS 26.442: "Codec for Enhanced Voice Services (EVS); ANSI C code (fixed-point)".

[15]	3GPP TS 26.443: "Codec for Enhanced Voice Services (EVS); ANSI C code (floating-point)".

[16]	3GPP TS 26.445: "Codec for Enhanced Voice Services (EVS); Detailed Algorithmic Description".

[17]	3GPP TS 26.446: "Codec for Enhanced Voice Services (EVS); Adaptive Multi-Rate - Wideband (AMR-WB) backward compatible functions".

[18]	3GPP TS 26.450: "Codec for Enhanced Voice Services (EVS); Discontinuous Transmission (DTX)".

[19]	3GPP TS 26.401: "General audio codec audio processing functions; Enhanced aacPlus general audio codec; General description".

[20]	3GPP TS 26.402: "General audio codec audio processing functions; Enhanced aacPlus general audio codec; Additional decoder tools".

[21]	3GPP TS 26.403: "General audio codec audio processing functions; Enhanced aacPlus general audio codec; Encoder specification; Advanced Audio Coding (AAC) part".

[22]	3GPP TS 26.404: "General audio codec audio processing functions; Enhanced aacPlus general audio codec; Enhanced aacPlus encoder Spectral Band Replication (SBR) part".

[23]	3GPP TS 26.405: "General audio codec audio processing functions; Enhanced aacPlus general audio codec; Encoder specification parametric stereo part".

[24]	3GPP TS 26.410: "General audio codec audio processing functions; Enhanced aacPlus general audio codec; Floating-point ANSI-C code".

[25]	3GPP TS 26.411: "General audio codec audio processing functions; Enhanced aacPlus general audio codec; Fixed-point ANSI-C code".

[26]	3GPP TS 26.290: "Audio codec processing functions; Extended Adaptive Multi-Rate - Wideband (AMR-WB+) codec; Transcoding functions".

[27]	3GPP TS 26.304: "Extended Adaptive Multi-Rate - Wideband (AMR-WB+) codec; Floating-point ANSI-C code".

[28]	3GPP TS 26.273: "ANSI-C code for the fixed-point Extended Adaptive Multi-Rate - Wideband (AMR-WB+) speech codec".

[29]	3GPP TS 26.244: "Transparent end-to-end streaming service; 3GPP file format (3GP)".

[30]	ISO/IEC 23000-19: "Information Technology Multimedia Application Format (MPEG-A) – Part 19: Common Media Application Format (CMAF) for segmented media".

[31]	ISO/IEC 23009-1:2022: "Information technology -- Dynamic adaptive streaming over HTTP (DASH) -- Part 1: Media presentation description and segment formats."

[32]	CTA-5003: "Web Application Video Ecosystem (WAVE): Device Playback Capabilities Specification" available here https://cdn.cta.tech/cta/media/media/resources/standards/pdfs/cta-5003-final.pdf.

[33]	3GPP TS 26.244: "Transparent end-to-end streaming service; 3GPP file format (3GP)".

[34]	3GPP TS 26.452: "Codec for Enhanced Voice Services (EVS); ANSI C code; Alternative fixed-point using updated basic operators".

[35]	3GPP TS 26.447: "Codec for Enhanced Voice Services (EVS); Error concealment of lost packets".

[36]	3GPP TS 26.511: "5G Media Streaming (5GMS); Profiles, Codecs and Formats".

[37]	ISO/IEC 23003-3: "Information technology — MPEG audio technologies — Part 3: Unified speech and audio coding".

[38]	ISO/IEC 23003-4: "Information technology — MPEG audio technologies — Part 4: Dynamic range control"

[39]	ISO/IEC 14496-12: "Information technology - Coding of audio-visual objects - Part 12: ISO base media file format"

[40]	ISO/IEC 14496-14 "Information technology – Coding of audio-visual objects – Part 14: MP4 file format".

[41]	3GPP TS 26.250: "Codec for Immersive Voice and Audio Services - General overview"

[42]	3GPP TS 26.252: "Codec for Immersive Voice and Audio Services - Test sequences"

[43]	3GPP TS 26.253: "Codec for Immersive Voice and Audio Services - Detailed Algorithmic Description incl. RTP payload format and SDP parameter definitions"

[44]	3GPP TS 26.254: "Codec for Immersive Voice and Audio Services - Rendering"

[45]	3GPP TS 26.255: "Codec for Immersive Voice and Audio Services - Error concealment of lost packets"

[46]	3GPP TS 26.256: "Codec for Immersive Voice and Audio Services - Jitter Buffer Management"

[47]	3GPP TS 26.251: "Codec for Immersive Voice and Audio Services; C code (fixed-point)"

[48]	3GPP TS 26.258: "Codec for Immersive Voice and Audio Services; C code (floating-point)"

[49]	ISO/IEC 14496-3: "Information technology — Coding of audio-visual objects — Part 3: Audio"

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Bitstream: A media bitstream that conforms to an audio/speech encoding format and certain Operation Point.

Media Profile: A combination of a Bitstream encapsulated into a media container suitable for 5G Media Streaming Delivery.

Operation Point: A collection of discrete combinations of different content formats and the encoding format.

Receiver: A receiver that can decode and render any bitstream that is conforming to a certain Operation Point.

Sender: An entity that can process and encode formats associated to an Operation Point.

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AAC	Advanced Audio Coding

ABR	Adaptive BitRate

AMR	Adaptive MultiRate

CMAF	Common Media Application Format

DASH	Dynamic Adaptive Streaming over HTTP

DRC	Dynamic Range Control

DTX	Discontinuous Transmission

EVS	Enhanced Voice Services

ISO BMFF	ISO Base Media File Format

HTTP	Hyper Text Transfer Protocol

SBR	Spectral Band Replication

URN	Universal Resource Name

WAVE	Web Application Video Ecosystem

# 4	Overview

The speech and audio media capabilities defined in this specification are primarily introduced in order to be used as content format in the context of 5G Media Streaming, but not restricted to this use case. Parameters for audio encoder/decoder, content format and transport are defined.

The present document defines:

-	Media decoding capabilities: the requirements for a receiver in terms of decoding

-	Media encoding capabilities: the requirements for a sender in terms of encoding

-	Operation Points: A collection of discrete combinations of different content formats and the encoding formats. Operation Points are supported by

-	Bitstream Requirements: A media bitstream that conforms to an audio or speech encoding format and certain Operation Point.

-	Receiver Requirements: A function that can decode and playback any Bitstream that is conforming to a certain Operation Point in real-time.

-	Sender Requirements: A function that can process and encode any Bitstream that is conforming to a certain Operation Point in real-time.

-	The integration of each Operation Point in 5G Media Streaming as defined in TS 26.501 [2] and TS 26.511 [36].

# 5	Media Capabilities

## 5.1	Introduction

This clause documents speech and audio media capabilities in terms of decoding capabilities.

## 5.2	Decoding Capabilities

The following speech media decoding capabilities are defined:

-	AMR: All decoding requirements for the AMR speech codec as specified in 3GPP TS 26.071 [3], 3GPP TS 26.090 [4], 3GPP TS 26.073 [5] and 3GPP TS 26.104 [6]) including all 8 modes and source-controlled rate operation ‎3GPP TS 26.093 [7].

-	AMR-WB: All decoding requirements for the AMR-WB codec as specified in 3GPP TS 26.171 ‎‎[8], 3GPP TS 26.190 ‎[9], 3GPP TS 26.173 ‎[10] and 3GPP TS 26.204 [11] including all 9 modes and source-controlled rate operation ‎3GPP TS 26.193 [12].

-	EVS: All decoding requirements for the EVS codec as specified in 3GPP TS 26.441 [13], 3GPP TS 26.445 [16], 3GPP TS 26.442 [14] and 3GPP TS 26.443 [15] as described below including functions for backwards compatibility with AMR-WB (3GPP TS 26.446 [17]) and discontinuous transmission (3GPP TS 26.450 [18]).

NOTE 1:	Speech media decoding capabilities with the IVAS codec are identical to audio media decoding capabilities with IVAS as defined below.

The following audio media decoding capabilities are defined:

-	eAAC+: All decoding requirements for the eAAC+ audio codec as specified in 3GPP TS 26.401 [19], 3GPP TS 26.402 [20], 3GPP TS 26.410 [24] and 3GPP TS 26.411 [25].

-	AMR-WB+: All decoding requirements for the AMR-WB+ audio codec as specified in 3GPP TS 26.290 ‎‎[26], 3GPP TS 26.304 ‎[27] and 3GPP TS 26.273 [28].

-	xHE-AAC stereo: All decoding requirements for the xHE-AAC stereo audio codec as specified in the MPEG-D USAC "Extended high efficiency AAC profile" as defined in ISO/IEC 23003-3 [37] as well as all processing requirements applicable to the MPEG-D DRC loudness control profile and to the dynamic range control profile, level 1 or higher, as specified in ISO/IEC 23003-4 [38].

NOTE 2: xHE-AAC® is a registered trademark of Fraunhofer in Germany and other countries and is used with Fraunhofer’s permission.

-	IVAS: All decoding and rendering requirements for the IVAS codec as specified in 3GPP TS 26.250 [41], TS 26.252 [42], TS 26.253 [43], TS 26.254 [44], TS 26.255 [45], TS 26.256 [46], TS 26.251 (fixed-point) [47], and TS 26.258 (floating-point) [48].

NOTE: The IVAS decoder supports decoding of streams encoded with EVS. Therefore, support of IVAS media decoding capabilities implies support of EVS media decoding capabilities.

-	AAC-ELDv2: the capability to decode MPEG-4 Low Delay AAC v2 Profile Level 2 bitstreams [x9] and to output it as 2-channel audio. Note that this profile contains the audio object types 23 (ER AAC LD), 39 (ER AAC ELD) and 44 (LD MPEG Surround).

-	AAC-ELDv2: the capability to decode MPEG-4 Low Delay AAC v2 Profile Level 2 bitstreams [x9] and to output it as 2-channel audio. Note that this profile contains the audio object types 23 (ER AAC LD), 39 (ER AAC ELD) and 44 (LD MPEG Surround).

## 5.3	Encoding Capabilities

The following speech media encoding capabilities are defined:

-	AMR: The encoding requirements for the AMR speech codec as specified in 3GPP TS 26.401 [19], clause 7, as well as 3GPP TS 26.403 [21], 3GPP TS 26.404 [22] and 3GPP TS 26.405 [23].

-	AMR-WB: The encoding requirements for the AMR-WB by one of the following methods:

-	according to 3GPP TS 26.173 ‎[10]

-	according to 3GPP TS 26.204 [11];

-	the AMR-WB IO mode according to TS 26.442 [14] and TS 26.443 [15],

-	the AMR-WB IO mode according to TS 26.452 [34].

-	EVS: The encoding requirements for the EVS codec by one of the following methods:

-	TS 26.442 [14] and TS 26.443 [15] encoding functions; or

-	TS 26.452 [34] encoding functions.

NOTE: Speech media encoding capabilities with the IVAS codec are identical to audio media encoding capabilities for IVAS as defined below.

The following audio media encoding capabilities are defined:

-	eAAC+: The encoding requirements for the AAC+ audio codec as specified 3GPP TS 26.401 [19], clause 7, as well as 3GPP TS 26.403 [21], 3GPP TS 26.404 [22] and 3GPP TS 26.405 [23].

-	AMR-WB+: The encoding requirements for the AMR-WB+ audio codec by one of the following methods

-	according to 3GPP TS 26.273 [28]; or

-	according to 3GPP TS 26.304 [27].

- 	xHE-AAC stereo: All encoding requirements for the xHE-AAC stereo audio codec as specified in the MPEG-D USAC "Baseline USAC" profile as defined in ISO/IEC 23003-3 [37] and with the additional requirements that all encoded media contains the required metadata sets conforming to the MPEG-D DRC loudness control profile or to the dynamic range control profile, level 1 or higher, as specified in ISO/IEC 23003-4 [38].

- 	IVAS: All encoding requirements for the IVAS codec as specified in 3GPP TS 26.250 [41], TS 26.252 [42], TS 26.253 [43], TS 26.251 (fixed-point) [47], and TS 26.258 (floating-point) [48].

NOTE: The IVAS encoder supports EVS encoding. Therefore support of IVAS media encoding capabilities implies support of EVS media encoding capabilities.

-	AAC-ELDv2: the capability to encode MPEG-4 Low Delay AAC v2 Profile Level 2 according to ISO/IEC 14496-3 [x9]. Note that this profile contains the audio object types 23 (ER AAC LD), 39 (ER AAC ELD) and 44 (LD MPEG Surround).

## 5.4	Definition of simultaneous encoding and decoding capabilities

Multi-instance encoding and decoding capabilities are defined as follows:

-	< Media-Cap>-<N>: the capability to support up to N simultaneous decoding or encoding instances, each supporting the decoding or encoding capability < Media-Cap >.

For example EVS-2 decoding capability is the capability to simultaneously support 2 decoders with EVS media capabilities according to clause 5.2

# 6	Operation Points

## 6.1	Introduction

The speech and audio Operation Points defined in this clause are primarily introduced in order to be used as content format in the context of 5G Media Streaming, but not restricted to this use case.

An operation point is a combination of rendering formats and media decoding capabilities.

For each Operation Point, Bitstream and Receiver requirements are detailed in the remainder of clause 6.

Table 6.1 provides an overview of the Operation Points defined in the present document.

Table 6.1: Speech and Audio Operation Points

| Operation Point name | Format Properties | Decoding and Encoding Capabilities | Reference |
| --- | --- | --- | --- |
| AMR speech | Sampling frequency: 8 kHz | AMR | 6.2.2 |
| AMR-WB speech | Sampling frequency: 16 kHz | AMR-WB | 6.2.3 |
| EVS mono | Sampling frequency: 8, 16, 32, 48 kHz | EVS | 6.2.4 |
| eAAC+ stereo | Sampling frequency: 32, 44.1, 48 kHz | eAAC+ | 6.3.2 |
| AMR-WB+ | Sampling frequency: 8, 16, 32, 48 kHz | AMR-WB+ | 6.3.3 |
| xHE-AAC stereo | Sampling frequency: 32, 44.1, 48 kHz | xHE-AAC stereo | 6.3.4 |
| IVAS | Sampling frequency: 16, 32, 48kHz | IVAS | 6.3.5 |

## 6.2	Speech Operation Points

### 6.2.1	Introduction

This clause defines speech operation points. For each operation point, the requirements for the bitstream as well as for the receiver are defined.

NOTE:	Speech operation relying on IVAS codec falls back to the audio operation point for IVAS codec as defined below under section 6.3.5.

### 6.2.2	AMR

#### 6.2.2.1	Bitstream Encoding Requirements

The following requirements apply to the AMR Operation Point.

-	The sampling frequency shall be 8 kHz.

-	The bitstream shall be encoded according to either 3GPP TS 26.073 [5] or 3GPP TS 26.104 [6].

Note that the bitstream produced by the AMR encoder consists of 20ms encoded speech frames.

#### 6.2.2.2	Receiver Requirements

Receivers conforming to the AMR Operation Point shall support the AMR speech media decoding capability according to clause 5.2 and shall support playback of the decoded signal.

#### 6.2.2.3	Sender Requirements

Senders conforming to the AMR Operation Point shall support the AMR speech media encoding capability according to clause 5.3 in real-time for any speech source format with sampling frequency 8kHz.

### 6.2.3	AMR-WB

#### 6.2.3.1	Bitstream Requirements

The following requirements apply to the AMR-WB Operation Point.

-	The sampling frequency shall be 16 kHz.

-	The bitstream shall be encoded by one of the following methods:

-	according to 3GPP TS 26.173 ‎[10]

-	according to 3GPP TS 26.204 [11];

-	the AMR-WB IO mode according to TS 26.442 [14] and TS 26.443 [15],

-	the AMR-WB IO mode according to TS 26.452 [34].

Note that the bitstream produced by the AMR-WB encoder consists of 20 ms encoded speech frames.

#### 6.2.3.2	Receiver Requirements

Receivers conforming to the AMR-WB Operation Point shall support the AMR-WB speech media decoding capability according to clause 5.2 and shall support playback of the decoded signal.

#### 6.2.3.3	Sender Requirements

Senders conforming to the AMR-WB Operation Point shall support the AMR-WB speech media encoding capability according to clause 5.3 in real-time for any speech source format with sampling frequency 16kHz.

### 6.2.4	EVS

#### 6.2.4.1	Bitstream Encoding Requirements

The following requirements apply to the EVS Operation Point:

-	The sampling frequency shall be one of the following: 8, 16, 32, 48 kHz.

-	The bitstream shall be encoded according to one of the following methods

-	TS 26.442 [14] and TS 26.443 [15] encoding functions; or

-	TS 26.452 [34] encoding functions.

Note that the bitstream produced by the EVS encoder consists of 20ms encoded speech frames.

#### 6.2.4.2	Receiver Requirements

Receivers conforming to the EVS Operation Point shall support the EVS speech media decoding capability according to clause 5.2 and shall support playback of the decoded signal.

#### 6.2.4.3	Sender Requirements

Senders conforming to the EVS Operation Point shall support the EVS speech media encoding capability according to clause 5.3 in real-time for any speech source format with sampling frequency 8, 16, 32, 48 kHz.

## 6.3	Audio Operation Points

### 6.3.1	Introduction

This clause defines audio operation points. For each operation point, the requirements for the bitstream as well as for the receiver are defined.

### 6.3.2	eAAC+ stereo

#### 6.3.2.1	Bitstream Encoding Requirements

The following requirements apply to the eAAC+ stereo Operation Point.

-	The sampling frequency shall be either 32 kHz, 44.1 kHz or 48 kHz.

-	The bitstream shall be encoded according to 3GPP TS 26.401 [19], clause 7, as well as 3GPP TS 26.403 [21], 3GPP TS 26.404 [22] and 3GPP TS 26.405 [23].

NOTE: 	The specified eAAC+ encoder consists of AAC-LC with additional tools that can be enabled (SBR, PS and more), see [19].

#### 6.3.2.2	Receiver Requirements

Receivers conforming to the eAAC+ stereo Operation Point shall support the eAAC+ media decoding capability according to clause 5.2 and shall support playback of the decoded signal.

NOTE: 	The eAAC+ decoder supports decoding of streams encoded with AAC-LC or aacPlus, see [19].

#### 6.3.2.3	Sender Requirements

Senders conforming to the eAAC+ stereo Operation Point shall support the eAAC+ stereo audio media encoding capability according to clause 5.3 in real-time for any stereo audio source format with sampling frequency 32kHz, 44.1kHz, 48kHz.

### 6.3.3	AMR-WB+

#### 6.3.3.1	Bitstream Encoding Requirements

The following requirements apply to the AMR-WB+ Operation Point.

-	The sampling frequency shall be either 8, 16, 32 or 48 kHz.

-	The bitstream shall be encoded by one of the following methods

-	according to 3GPP TS 26.273 [28]; or

-	according to 3GPP TS 26.304 [27].

#### 6.3.3.2	Receiver Requirements

Receivers conforming to the AMR-WB+ Operation Point shall support the AMR-WB+ media decoding capability according to clause 5.2 and shall support playback of the decoded signal.

#### 6.3.3.3	Sender Requirements

Senders conforming to the AMR-WB+ Operation Point shall support the AMR-WB+ audio media encoding capability according to clause 5.3 in real-time for any stereo audio source format with sampling frequency 8, 16, 32 or 48 kHz.

### 6.3.4	xHE-AAC stereo

#### 6.3.4.1	Bitstream Encoding Requirements

The following requirements apply to the xHE-AAC stereo Operation Point.

-	The sampling frequency shall be either 32 kHz, 44.1 kHz or 48 kHz.

-	The bitstream shall be encoded according to the MPEG-D USAC "Baseline USAC" profile as defined in ISO/IEC 23003-3 [37] and shall contain the metadata sets conforming to the MPEG-D DRC loudness control profile or to the dynamic range control profile, level 1 or higher, as specified in ISO/IEC 23003-4 [38].

#### 6.3.4.2	Receiver Requirements

Receivers conforming to the xHE-AAC stereo Operation Point shall support the xHE-AAC stereo media decoding capability according to clause 5.2 and shall support playback of the decoded signal.

NOTE: 	The xHE-AAC decoder supports decoding of streams encoded with eAAC+ [19]. Therefore support of xHE-AAC stereo media decoding capabilities implies support of eAAC+ media decoding capabilities.

#### 6.3.4.3	Sender Requirements

Senders conforming to the xHE-AAC stereo Operation Point shall support the xHE-AAC audio media encoding capability according to clause 5.3 in real-time for any stereo audio source format with sampling frequency 32kHz, 44.1kHz, or 48kHz.

### 6.3.5	IVAS

#### 6.3.5.1	Bitstream Encoding Requirements

The following requirements apply to the IVAS Operation Point.

-	The input audio format shall be either mono, stereo, binaural, multi-channel (5.1, 5.1.2, 5.1.4, 7.1, 7.1.4), scene-based (Ambisonics up to 3rd order), metadata-assisted spatial audio (MASA), object-based, a combined format of objects with scene-based (OSBA), or a combined format of objects with metadata-assisted spatial audio (OMASA).

-	The sampling frequency shall be either 8 kHz (only EVS interoperable coding), 16 kHz, 32 kHz and 48 kHz (fullband audio content).

-	The bitstream shall be encoded according to 3GPP TS 26.250 [41], TS 26.252 [42], TS 26.253 [43], TS 26.251 (fixed-point) [47], and TS 26.258 (floating-point) [48].

NOTE:	IVAS codec level setting is TBD.

#### 6.3.5.2	Receiver Requirements

Receivers conforming to the IVAS Operation Point shall support the IVAS media decoding capability according to clause 5.2 and shall support rendering and playback of the decoded signal.

NOTE: The IVAS decoder supports decoding of streams encoded with EVS. Therefore support of IVAS media decoding capabilities implies support of EVS media decoding capabilities.

NOTE: IVAS codec level setting is TBD.

#### 6.3.5.3	Sender Requirements

Senders conforming to the IVAS Operation Point shall support the IVAS-Enc media encoding capability according to clause 5.3 in real-time for the audio formats according to the supported IVAS codec level 1, 2 or 3 as either mono, stereo, binaural, multi-channel (5.1, 5.1.2, 5.1.4, 7.1, 7.1.4), scene-based (Ambisonics up to 3rd order), metadata-assisted spatial audio (MASA), and object-based with sampling frequency 8 kHz (only EVS interoperable coding), 16 kHz, 32 kHz and 48 kHz (fullband audio content).

NOTE: IVAS codec level setting is TBD.

### 6.3.6	AAC-ELDv2

#### 6.3.6.1	Bitstream Encoding Requirements

The following requirements apply to the AAC-ELDv2 Operation Point.

-	The sampling frequency shall be either 32 kHz, 44.1 kHz or 48 kHz.

-	The bitstream shall be encoded according to MPEG-4 Low Delay AAC v2 Profile Level 2 as specified in ISO/IEC 14496-3 [x9].

#### 6.3.6.2	Receiver Requirements

Receivers conforming to the AAC-ELDv2 Operation Point shall support the AAC-ELDv2 media decoding capability according to clause 5.2 and shall support playback of the decoded signal.

#### 6.3.6.3	Sender Requirements

Senders conforming to the AAC-ELDv2 Operation Point shall support the AAC-ELDv2 audio media encoding capability according to clause 5.3 in real-time for any stereo audio source format with sampling frequency 32kHz, 44.1kHz, or 48kHz.

# 7	Mapping to 5GMS delivery

## 7.1	Introduction

This clause defines the mapping of the Operation Points as defined in clause 6 to 5G Media Streaming delivery. In particular the following aspects are addressed:

-	Encapsulation of a bitstream into an ISO BMFF track.

-	Definition of media content and receivers conforming to media profile including:

-	Encapsulation of a bitstream into a CMAF track

-	Providing the content in a CMAF Switching Set

-	Mapping to DASH-based distribution

-	Playback Requirements for a receiver conforming to this media profile

## 7.2	AMR Media Profile

### 7.2.1	Mapping to ISO BMFF

If media is provided following the operation point AMR and is encapsulated in the ISO BMFF [39], then the file format track shall contain the sample entry AMRSampleEntry with box_type 'samr' as defined in TS 26.244 [29] clause 6.5 and conform to the requirements of the sample entry 'samr' as defined in TS 26.244 [29].

### 7.2.2	Media Profile Definition

#### 7.2.2.1	CMAF Track Definition

If media is provided following the operation point AMR and is encapsulated in a CMAF track, then the CMAF track shall conform to clause 7.2.1, and conform to the general CMAF Track constraints in ISO/IEC 23000-19 [30] as well as the general audio track constraints defined in ISO/IEC 23000-19 [30], clause 10.

#### 7.2.2.2	CMAF Switching Set and Media Profile Definition

If media is provided following the operation point AMR and is provided in a CMAF Switching Set, then every CMAF track in the CMAF Switching Set shall conform to clause 7.2.2.1, and shall conform to the general CMAF Switching Set constraints in ISO/IEC 23000-19 [30], clause 7. CMAF Switching Sets that follow these requirements conform to the CMAF AMR media profile 'camr' defined in this clause.

#### 7.2.2.3	Mapping to DASH Adaptation Set

If media is provided following the operation point AMR, a CMAF Switching Set conforming to clause 7.2.2.2 may be provided in a DASH Media Presentation Description in an Adaptation Set, in that case, the Adaptation Set shall conform to the Adaptation Set constraints of the DASH profile for CMAF as defined in ISO/IEC 23009-1 [31]. The following parameters shall be present on Adaptation Set level and set:

-	@codecs is set to 'samr'

-	@mimeType is set to be compatible with "audio/mp4 profiles='camr'"

-	@audioSamplingRate is set to '8000'

If the Adaptation Set conforms to the constraints for the AMR Operation Point as defined in this clause, then the @profiles parameter in the Adaptation Set may signal conformance to this Media Profile by using "urn:3GPP:audio:mp:amr".

#### 7.2.2.4	Playback Requirements

For a receiver supporting the AMR media profile the following applies:

-	It shall support the receiver requirements as documented in clause 6.2.2.2 for any CMAF Track conforming to the CMAF AMR media profile 'camr' as defined in clause 7.2.2.1.

-	It shall support the following playback requirements as documented in clause 8 of CTA-WAVE 5003 [32] for any content conforming to a CMAF Switching Set according to CMAF AMR media profile 'camr' as defined in clause 7.2.2.2, namely:

-	8.2 Sequential Track Playback

-	8.3	Random Access to Fragment

-	8.4 Random Access to Time

-	8.5 Switching Set Playback

-	8.6 Regular Playback of Chunked Content

-	8.7 Regular Playback of Chunked Content, non-aligned append

-	It should support the following playback requirements as documented in clause 8 of CTA-WAVE 5003 [29] for any content conforming to a CMAF Switching Set according to CMAF AMR media profile 'camr' as defined in clause 7.2.2.2, namely:

-	8.9 Out-Of-Order Loading

-	8.10 Overlapping Fragments

-	8.12 Playback of Encrypted Content

#### 7.2.2.5	Content Generation Requirements

For a transmitter supporting the AMR media profile the following applies:

-	It shall support all media encoding capabilities for AMR as defined in clause 5.3.

-	It shall support the sender requirements for AMR as defined in clause 6.2.2.3.

-	It shall support the generation of a CMAF Track as defined in clause 7.2.2.1 that conforms to the CMAF Media Profile 'camr' as defined in clause 7.2.2.2.

-	If used for Adaptive Bit Rate (ABR) distribution, it shall support the generation of a CMAF Switching Set as defined in clause 7.2.2.4.

## 7.3	AMR-WB Media Profile

### 7.3.1	Mapping to ISO BMFF

If media is provided following the operation point AMR-WB and is encapsulated in the ISO BMFF, then the file format track shall contain the AMRSampleEntryBox and shall conform to the requirements of the sample entry 'sawb' as defined in TS 26.244 [29] clause 6.5.

### 7.3.2	Media Profile Definition

#### 7.3.2.1	CMAF Track Definition

If media is provided following the operation point AMR-WB and is encapsulated in a CMAF track, then the CMAF track shall conform to clause 7.3.1, and conform to the general CMAF Track constraints in ISO/IEC 23000-19 [30], clause 7 as well as the general audio track constraints defined in ISO/IEC 23000-19 [30], clause 10.

#### 7.3.2.2	CMAF Switching Set and Media Profile Definition

If media is provided following the operation point AMR-WB and is provided in a CMAF Switching Set, then every CMAF track in the CMAF Switching Set shall conform to clause 7.3.2.1, and shall conform to the general CMAF Switching Set constraints in ISO/IEC 23000-19 [30], clause 7. CMAF Switching Sets that follow these requirements conform to the CMAF AMR WB media profile 'camw' defined in this clause.

#### 7.3.2.3	Mapping to DASH Adaptation Set

If media is provided following the operation point AMR-WB, a switching set conforming to clause 7.3.2.2 may be provided in a DASH Media Presentation Description in an Adaptation Set, in that case, the Adaptation Set shall conform to the Adaptation set constraints of the DASH profile for CMAF as defined in ISO/IEC 23009-1 [31] clause 8.12.4.3. The following parameters shall be present on Adaptation Set level:

-	@codecs is set to 'sawb'

-	@mimeType is set to be compatible with "audio/mp4 profiles='camw'"

-	@audioSamplingRate is set to '16000'

If the Adaptation Set conforms to the constraints for the AMR-WB Operation Point as defined in this clause, then the @profiles parameter in the Adaptation Set may signal conformance to this Media Profile by using "urn:3GPP:audio:mp:amr-wb".

#### 7.3.2.4	Playback Requirements

For a receiver supporting the AMR-WB media profile the following applies:

-	It shall support the receiver requirements as documented in clause 6.2.3.2 for any CMAF Track conforming to the CMAF AMR-WB media profile 'camw' as defined in clause 7.3.2.2.

-	It shall support the following playback requirements as documented in clause 8 of CTA-WAVE 5003 [32] for any content conforming to a CMAF Switching Set according to CMAF AMR-WB media profile 'camw' as defined in clause 7.3.2.2, namely:

-	8.2 Sequential Track Playback

-	8.3	Random Access to Fragment

-	8.4 Random Access to Time

-	8.5 Switching Set Playback

-	8.6 Regular Playback of Chunked Content

-	8.7 Regular Playback of Chunked Content, non-aligned append

-	It should support the following playback requirements as documented in clause 8 of CTA-WAVE 5003 [32] for any content conforming to a CMAF Switching Set according to CMAF AMR-WB media profile 'camw' as defined in clause 7.3.2.2, namely:

-	8.9 Out-Of-Order Loading

-	8.10 Overlapping Fragments

-	8.12 Playback of Encrypted Content

#### 7.3.2.5	Content Generation Requirements

For a transmitter supporting the AMR-WB media profile the following applies:

-	It shall support all media encoding capabilities for AMR-WB as defined in clause 5.3.

-	It shall support the sender requirements for AMR-WB as defined in clause 6.2.3.3.

-	It shall support the generation of a CMAF Track as defined in clause 7.3.2.1 that conforms to the CMAF Media Profile 'camw' as defined in clause 7.3.2.2.

-	If used for Adaptive Bit Rate (ABR) distribution, it shall support the generation of a CMAF Switching Set as defined in clause 7.3.2.4.

## 7.4	EVS Media Profile

### 7.4.1	Mapping to ISO BMFF

If media is provided following the operation point EVS and is encapsulated in the ISO BMFF, then the file format track shall contain the EVSSampleEntryBox with box_type and conform to the requirements of the sample entry 'sevs' as defined in TS 26.244 [29].

### 7.4.2	Media Profile Definition

#### 7.4.2.1	CMAF Track Definition

If media is provided following the operation point EVS and is encapsulated in a CMAF track, then the CMAF track shall conform to clause 7.3.1, and conform to the general CMAF Track constraints in ISO/IEC 23000-19 [30], clause 7 as well as the general audio track constraints defined in ISO/IEC 23000-19 [30], clause 10.

#### 7.4.2.2	CMAF Switching Set and Media Profile Definition

If media is provided following the operation point EVS and is provided in a CMAF Switching Set, then every CMAF track in the CMAF Switching Set shall conform to clause 7.4.2.1, and shall conform to the general CMAF Switching Set constraints in ISO/IEC 23000-19 [30], clause 7. A CMAF Switching Set that follows these requirements conforms to the CMAF EVS media profile 'cevs' defined in this clause.

#### 7.4.2.3	Mapping to DASH Adaptation Set

If media is provided following the operation point EVS and is provided in a DASH Media Presentation in an Adaptation Set, a switching set conforming to clause 7.4.2.2 may be provided in a DASH Media Presentation Description in an Adaptation Set. In that case, the Adaptation Set shall conform to the Adaptation set constraints of the DASH profile for CMAF as defined in ISO/IEC 23009-1 [31]. The following parameters shall be present on Adaptation Set level:

-	@codecs is set to 'sevs'

-	@mimeType is set to be compatible with "audio/mp4 profiles='cevs'"

-	@audioSamplingRate is set to one of the following: '8000', '16000', '24000', '32000'

If the Adaptation Set conforms to the constraints for the EVS Operation Point as defined in this clause, then the @profiles parameter in the Adaptation Set may signal conformance to this Media Profile by using "urn:3GPP:audio:mp:evs.

#### 7.4.2.4	Playback Requirements

For a receiver supporting the EVS media profile the following applies:

-	It shall support the receiver requirements as documented in clause 6.2.4.2 for any CMAF Track conforming to the CMAF EVS media profile 'cevs' as defined in clause 7.4.2.2.

-	It shall support the following playback requirements as documented in clause 8 of CTA-WAVE 5003 [32] for any content conforming to a CMAF Switching Set according to CMAF EVS media profile 'cevs' as defined in clause 7.4.2.2, namely:

-	8.2 Sequential Track Playback

-	8.3	Random Access to Fragment

-	8.4 Random Access to Time

-	8.5 Switching Set Playback

-	8.6 Regular Playback of Chunked Content

-	8.7 Regular Playback of Chunked Content, non-aligned append

-	It should support the following playback requirements as documented in clause 8 of CTA-WAVE 5003 [32] for any content conforming to a CMAF Switching Set according to CMAF EVS media profile 'cevs' as defined in clause 7.2.2.2, namely:

-	8.9 Out-Of-Order Loading

-	8.10 Overlapping Fragments

-	8.12 Playback of Encrypted Content

#### 7.4.2.5	Content Generation Requirements

For a transmitter supporting the EVS media profile the following applies:

-	It shall support all media encoding capabilities for EVS as defined in clause 5.3.

-	It shall support the sender requirements for EVS as defined in clause 6.2.4.3.

-	It shall support the generation of a CMAF Track as defined in clause 7.4.2.1 that conforms to the CMAF Media Profile 'cevs' as defined in clause 7.4.2.2.

-	If used for Adaptive Bit Rate (ABR) distribution, it shall support the generation of a CMAF Switching Set as defined in clause 7.4.2.2.

## 7.5	IVAS Media Profile

### 7.5.1	Mapping to ISO BMFF

If media is provided following the operation point IVAS and is encapsulated in the ISO BMFF, then the file format track shall contain the IVASSampleEntryBox with box_type and conform to the requirements of the sample entry 'sivs' as defined in TS 26.244 [29].

### 7.5.2	Media Profile Definition

#### 7.5.2.1	CMAF Track Definition

If media is provided following the operation point IVAS and is encapsulated in a CMAF track, then the CMAF track shall conform to clause 7.5.1, and conform to the general CMAF Track constraints in ISO/IEC 23000-19 [30], clause 7 as well as the general audio track constraints defined in ISO/IEC 23000-19 [30], clause 10.

#### 7.5.2.2	CMAF Switching Set and Media Profile Definition

If media is provided following the operation point IVAS and is provided in a CMAF Switching Set, then every CMAF track in the CMAF Switching Set shall conform to clause 7.5.2.1, and shall conform to the general CMAF Switching Set constraints in ISO/IEC 23000-19 [30], clause 7. A CMAF Switching Set that follows these requirements conforms to the CMAF IVAS media profile 'civs' defined in this clause.

#### 7.5.2.3	Mapping to DASH Adaptation Set

If media is provided following the operation point IVAS and is provided in a DASH Media Presentation in an Adaptation Set, a switching set conforming to clause 7.5.2.2 may be provided in a DASH Media Presentation Description in an Adaptation Set. In that case, the Adaptation Set shall conform to the Adaptation set constraints of the DASH profile for CMAF as defined in ISO/IEC 23009-1 [31]. The following parameters shall be present on Adaptation Set level:

-	@codecs is set to 'sivs'

-	@mimeType is set to be compatible with "audio/mp4 profiles='civs'"

-	@audioSamplingRate is set to one of the following: '16000', '32000', '48000'

If the Adaptation Set conforms to the constraints for the IVAS Operation Point as defined in this clause, then the @profiles parameter in the Adaptation Set may signal conformance to this Media Profile by using "urn:3GPP:audio:mp:ivas.

#### 7.5.2.4	Playback Requirements

For a receiver supporting the IVAS media profile the following applies:

-	It shall support the receiver requirements as documented in clause 6.3.5.2 for any CMAF Track conforming to the CMAF IVAS media profile 'civs' as defined in clause 7.5.2.2.

-	It shall support the following playback requirements as documented in clause 8 of CTA-WAVE 5003 [32] for any content conforming to a CMAF Switching Set according to CMAF IVAS media profile 'civs' as defined in clause 7.5.2.2, namely:

-	8.2 Sequential Track Playback

-	8.3	Random Access to Fragment

-	8.4 Random Access to Time

-	8.5 Switching Set Playback

-	8.6 Regular Playback of Chunked Content

-	8.7 Regular Playback of Chunked Content, non-aligned append

-	It should support the following playback requirements as documented in clause 8 of CTA-WAVE 5003 [32] for any content conforming to a CMAF Switching Set according to CMAF IVAS media profile 'civs' as defined in clause 7.5.2.2, namely:

-	8.9 Out-Of-Order Loading

-	8.10 Overlapping Fragments

-	8.12 Playback of Encrypted Content

#### 7.5.2.5	Content Generation Requirements

For a transmitter supporting the IVAS media profile the following applies:

-	It shall support all media encoding capabilities for IVAS as defined in clause 5.3.

-	It shall support the sender requirements for IVAS as defined in clause 6.3.5.3.

-	It shall support the generation of a CMAF Track as defined in clause 7.5.2.1 that conforms to the CMAF Media Profile 'civs' as defined in clause 7.5.2.2.

-	If used for Adaptive Bit Rate (ABR) distribution, it shall support the generation of a CMAF Switching Set as defined in clause 7.5.2.2.

## 7.6	eAAC+ stereo Media Profile

### 7.6.1	Void

### 7.6.2	Media Profile Definition

#### 7.6.2.1	CMAF Track Definition

If media is provided following the operation point eAAC+ stereo and is encapsulated in a CMAF track, then the CMAF track shall conform to and contain the MP4AudioSampleEntry 'mp4a' as defined in ISO/IEC 14496-14 [40], the general CMAF Track constraints in ISO/IEC 23000-19 [30], clause 7, the general audio track constraints defined in ISO/IEC 23000-19 [30], clause 10 as well as AAC core constraints in clause 10 of ISO/IEC 23000-19 [30].

NOTE: 	A CMAF Track conforming to eAAC+ stereo media profile also conforms to the xHE-AAC Media Profile as defined in clause 7.8. Consequently, such CMAF Track can also be played back by receivers conforming to the xHE-AAC Media Profile.

#### 7.6.2.2	CMAF Switching Set and Media Profile Definition

If media is provided following the operation point eAAC+ stereo and is provided in a CMAF Switching Set, then every CMAF track in the CMAF Switching Set shall conform to clause 7.6.2.1, and the tracks shall conform to the general CMAF Switching Set constraints in ISO/IEC 23000-19 [30], clause 7 as well as the AAC core Switching Set constraints in clause 10.5 of ISO/IEC 23000-19 [30]. A CMAF Switching Set following these requirements is defined as the CMAF eAAC+ stereo media profile 'ceac'.

#### 7.6.2.3	Mapping to DASH Adaptation Set

If media is provided following the operation point eAAC+ stereo, a switching set conforming to clause 7.6.2.2 may be provided in a DASH Media Presentation Description in an Adaptation Set. In that case, the Adaptation Set shall conform to the Adaptation set constraints of the DASH profile for CMAF as defined in ISO/IEC 23009-1 [31] clause 8.12.4.3. The following parameters shall be present on Adaptation Set level and set:

-	@codecs is set to 'mp4a.40.29'

-	@mimeType is set to be compatible with "audio/mp4; profiles='ceac'"

-	@audioSamplingRate is set to '32000','44100', or '48000'

If the Adaptation Set conforms to the constraints for the eAAC+ stereo Operation Point as defined in this clause, then the @profiles parameter in the Adaptation Set may signal conformance to this Media Profile by using "urn:3GPP:audio:mp:eAAC+ ".

NOTE: 	A DASH Adaptation Set conforming to eAAC+ stereo media profile conforms to the xHE-AAC DASH Adaptation Set as defined in clause 7.8. Hence, such DASH Adaptation Set can also be played back by receivers conforming to the xHE-AAC media profile.

#### 7.6.2.4	Playback Requirements

For a receiver supporting the eAAC+ stereo media profile the following applies:

-	It shall support the receiver requirements as documented in clause 6.3.2.2 for any CMAF Track conforming to the CMAF eAAC+ stereo media profile 'ceac' as defined in clause 7.6.2.2.

-	It shall support the following playback requirements as documented in clause 8 of CTA-WAVE 5003 [32] for any content conforming to a CMAF Switching Set according to CMAF eAAC+ stereo media profile 'ceac' as defined in clause 7.6.2.2, namely:

-	8.2 Sequential Track Playback

-	8.3	Random Access to Fragment

-	8.4 Random Access to Time

-	8.5 Switching Set Playback

-	8.6 Regular Playback of Chunked Content

-	8.7 Regular Playback of Chunked Content, non-aligned append

-	It should support the following playback requirements as documented in clause 8 of CTA-WAVE 5003 [32] for any content conforming to a CMAF Switching Set according to CMAF eAAC+ stereo media profile 'ceac' as defined in clause 7.6.2.2, namely:

-	8.9 Out-Of-Order Loading

-	8.10 Overlapping Fragments

-	8.12 Playback of Encrypted Content

#### 7.6.2.5	Content Generation Requirements

For a transmitter supporting the eAAC+ stereo media profile the following applies:

-	It shall support all media encoding capabilities for eAAC+ stereo as defined in clause 5.3.

-	It shall support the sender requirements for eAAC+ stereo as defined in clause 6.3.2.3.

-	It shall support the generation of a CMAF Track as defined in clause 7.6.2.1 that conforms to the CMAF Media Profile 'ceac' as defined in clause 7.6.2.2.

-	If used for Adaptive Bit Rate (ABR) distribution, it shall support the generation of a CMAF Switching Set as defined in clause 7.6.2.2.

## 7.7	AMR-WB+ Media Profiles

### 7.7.1	Mapping to ISO BMFF

If media is provided following the operation point AMR-WB+ and is encapsulated in the ISO BMFF, then the file format track shall contain the AMRWPSampleEntry with box_type and conform to the requirements of the sample entry 'sawp' as defined in TS 26.244 [29] clause 6.9.

### 7.7.2	Media Profile Definition

#### 7.7.2.1	CMAF Track Definition

If media is provided following the operation point AMR-WB+ and is encapsulated in a CMAF track, then the CMAF track shall conform to clause 7.7.1 and the general CMAF Track constraints in ISO/IEC 23000-19 [30], clause 7 as well as the general audio track constraints defined in ISO/IEC 23000-19, clause 10.

#### 7.7.2.2	CMAF Switching Set and Media Profile Definition

If media is provided following the operation point AMR-WB+ and is provided in a CMAF Switching Set, then every CMAF track in the CMAF Switching Set shall conform to the requirements of clause 7.7.2.1 and the general CMAF Switching Set constraints in ISO/IEC 23000-19 [30], clause 7. A CMAF Switching Set following these requirements is defined as the CMAF AMR-WB+ media profile 'camp'.

#### 7.7.2.3	Mapping to DASH Adaptation Set

If media is provided following the operation point AMR-WB+, a switching set conforming to clause 7.7.2.2 may be provided in a DASH Media Presentation in an Adaptation Set. In that case, the Adaptation Set shall conform to the Adaptation set constraints of the DASH profile for CMAF as defined in ISO/IEC 23009-1 [31] clause 8.12.4.3. The following parameters shall be present on Adaptation Set level:

-	@codecs is set to 'sawp'

-	@mimeType is set to be compatible with "audio/mp4 profiles='camp'"

-	@audioSamplingRate is set to any of the following values: '8000', '16000', '32000', or '38400'

If the Adaptation Set conforms to the constraints for the AMR-WB+ Operation Point as defined in this clause, then the @profiles parameter in the Adaptation Set may signal conformance to this Media Profile by using "urn:3GPP:audio:mp:amr-wb+".

#### 7.7.2.4	Playback Requirements

For a receiver supporting the AMR-WB+ media profile the following applies:

-	It shall support the receiver requirements as documented in clause 6.3.2.2 for any CMAF Track conforming to the CMAF AMR-WB+ stereo media profile 'camp' as defined in clause 7.7.2.2.

-	It shall support the following playback requirements as documented in clause 8 of CTA-WAVE 5003 [32] for any content conforming to a CMAF Switching Set according to CMAF AMR-WB+ media profile 'camp' as defined in clause 7.7.2.2, namely:

-	8.2 Sequential Track Playback

-	8.3	Random Access to Fragment

-	8.4 Random Access to Time

-	8.5 Switching Set Playback

-	8.6 Regular Playback of Chunked Content

-	8.7 Regular Playback of Chunked Content, non-aligned append

-	It should support the following playback requirements as documented in clause 8 of CTA-WAVE 5003 [32] for any content conforming to a CMAF Switching Set according to CMAF AMR-WB media profile 'camp' as defined in clause 7.7.2.2, namely:

-	8.9 Out-Of-Order Loading

-	8.10 Overlapping Fragments

-	8.12 Playback of Encrypted Content

#### 7.7.2.5	Content Generation Requirements

For a transmitter supporting the AMR-WB+ media profile the following applies:

-	It shall support all media encoding capabilities for AMR-WB+ stereo as defined in clause 5.3.

-	It shall support the sender requirements for AMR-WB+ as defined in clause 6.3.3.3.

-	It shall support the generation of a CMAF Track as defined in clause 7.7.2.1 that conform to the CMAF Media Profile 'cawp' as defined in clause 7.7.2.2.

-	If used for Adaptive Bit Rate (ABR) distribution, it shall support the generation of a CMAF Switching Set as defined in clause 7.7.2.2.

## 7.8	xHE-AAC Media Profile

### 7.8.1	CMAF Track Definition

If media is provided following the operation point xHE-AAC stereo and is encapsulated in a CMAF track, then the CMAF track shall conform to the general CMAF Track constraints in ISO/IEC 23000-19:2023 [30], clause 7, the general audio track constraints defined in ISO/IEC 23000-19 [30], clause 10, as well as MPEG-D USAC track format constraints defined in ISO/IEC 23000-19 [30], Annex K. As specified in ISO/IEC 23000-19 [30], Annex K, the MP4AudioSampleEntry shall be present with name `mp4a`.

### 7.8.2	CMAF Switching Set and Media Profile Definition

If media is provided following the operation point xHE-AAC stereo and is provided in a CMAF Switching Set, then every CMAF track in the CMAF Switching Set shall conform to clause 7.8.1 and the tracks shall conform to the general CMAF Switching Set constraints in ISO/IEC 23000-19 [30], clause 7 as well as the AAC core Switching Set constraints in clause 10.5 of ISO/IEC 23000-19 [30].

A CMAF Switching Set following these requirements is defined as the CMAF USAC stereo media profile 'casu'.

### 7.8.3	Mapping to DASH Adaptation Set

If media is provided following the operation point xHE-AAC stereo and is provided in a DASH Media Presentation in an Adaptation Set, then the following requirements apply:

-	Content representations shall comply with the USAC stereo CMAF media profile 'casu', as defined in ISO/IEC 23000-19 [30] annex K.

- 	Content representations shall comply with the DASH Profile for CMAF content as defined in ISO/IEC 23009-1:2022 [31]

-	If the ChannelConfiguration parameter is present in the Movie header, then the identical channel configuration shall be signaled by means of the AudioChannelConfiguration element in the MPD, according to the values specified in ISO/IEC 23000-19 [30] Table K.2.

-	@codecs is set to "mp4a.40.42"

-	@mimeType is set to be compatible with "audio/mp4; profiles='casu'"

-	@audioSamplingRate is set to '32000','44100', or '48000'

If the Adaptation Set conforms to the constraints for the xHE-AAC stereo Operation Point as defined in this clause, then the @profiles parameter in the Adaptation Set may signal conformance to this Media Profile by using "urn:3GPP:audio:mp:xHE-AAC".

### 7.8.4	Playback Requirements

For a receiver supporting the xHE-AAC media profile the following applies:

-	It shall support the receiver requirements as documented in clause 6.3.4.2 for any CMAF Track conforming to the CMAF USAC stereo media profile 'casu' as defined in clause 7.8.2.

-	It should support the audio related playback requirements as documented in clause 8 of CTA-WAVE 5003 [32] for any content conforming to a CMAF Switching Set according to CMAF USAC stereo media profile 'casu' as defined in clause 7.8.2.

### 7.8.5	Content Generation Requirements

For a transmitter supporting the xHE-AAC media profile the following applies:

-	It shall support all media encoding capabilities for xHE-AAC stereo as defined in clause 5.3.

-	It shall support the sender requirements for xHE-AAC stereo as defined in clause 6.3.4.3.

-	It shall support the generation of a CMAF Track as defined in clause 7.8.1 that conforms to the CMAF Media Profile 'casu' as defined in clause 7.8.2.

-	If used for Adaptive Bit Rate (ABR) distribution, it shall support the generation of a CMAF Switching Set as defined in clause 7.8.2.

Annex A (informative):
Registration Information

# A.1	3GPP Registered URIs

The clause documents the registered URIs in the present document following the process in http://www.3gpp.org/specifications-groups/34-uniform-resource-name-urn-list

Table A-1 lists all registered URN values as well as:

-	a brief description of its functionality;

-	a reference to the specification or other publicly available document (if any) containing the definition;

-	the name and email address of the person making the application; and

-	any supplementary information considered necessary to support the application.

Table A-1: 3GPP Registered URNs

| URN | Description | Reference | Contact | Remarks |
| --- | --- | --- | --- | --- |
| urn:3GPP:audio:mp:amr | AMR Media Profile | TS 26.117, clause 7.2.2.3 | Thomas Stockhammer tsto@qti.qualcomm.com | none |
| urn:3GPP:audio:mp:amr-wb | AMR-WB Media Profile | TS 26.117, clause 7.3.2.3 | Thomas Stockhammer tsto@qti.qualcomm.com | none |
| urn:3GPP:audio:mp:evs | EVS Media Profile | TS 26.117, clause 7.4.2.3 | Thomas Stockhammer tsto@qti.qualcomm.com | none |
| urn:3GPP:audio:mp:ivas | IVAS Media Profile | TS 26.117 clause 7.5.2.3 | tbd | none |
| urn:3GPP:audio:mp:eAAC+ | eAAC+ stereo Media Profile | TS 26.117, clause 7.6.2.3 | Thomas Stockhammer tsto@qti.qualcomm.com | none |
| urn:3GPP:audio:mp:amr-wb+ | AMR-WB+ Media Profile | TS 26.117, clause 7.7.2.3 | Thomas Stockhammer tsto@qti.qualcomm.com | none |
| urn:3GPP:audio:mp:xHE-AAC | xHE-AAC Media Profile | TS 26.117, clause 7.8.3 | Frédéric Gabin frederic.gabin@dolby.com | none |

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-12 | SA#86 | SP-190990 |  |  |  | Presented to TSG SA#86 (for information) | 1.0.0 |
| 2020-01 | SA4#107 | S4-200245 |  |  |  | Version agreed by TSG SA WG 4 to be sent for SA approval | 1.1.0 |
| 2020-03 | SA#87-e | SP-200049 |  |  |  | Presented to TSG SA#87-e (for approval) | 2.0.0 |
| 2020-03 | SA#87-e | SP-200049 |  |  |  | Approved by TSG SA#87-e | 16.0.0 |
| 2022-04 | - | - | - | - | - | Update to Rel-17 version (MCC) | 17.0.0 |
| 2023-03 | SA#99- | SP-230251 | 0001 | 4 | B | Introduction of xHE-AAC stereo audio operation point | 18.0.0 |
| 2023-06 | SA#100 | SP-230552 | 0002 | - | F | Corrections to references | 18.1.0 |
| 2024-03 | SA#103 | SP-240462 | 0008 | 4 | A | [5GMS3] Correction on Audio encapsulation in ISO BMFF, 3GP file format, CMAF and DASH | 18.2.0 |
| 2024-03 | SA#103 | SP-240046 | 0005 | 4 | B | On MeCAR Audio capabilities | 18.2.0 |
| 2024-06 | SA#104 | SP-240694 | 0009 | 2 | F | Adding missing media profile | 18.3.0 |
| 2025-03 | SA#107 | SP-250129 | 0010 | 1 | F | Correction to HE-AAC and xHE-AAC stereo mapping to DASH adaptation set | 18.4.0 |
| 2025-10 | - | - | - | - | - | Update to Rel-19 version (MCC) | 19.0.0 |
| 2025-12 | SA#110 | SP-251435 | 0011 | 1 | B | Updates related to availability of TS 26.251 (IVAS fixed-point code) | 19.1.0 |