| 3GPP TS 38.133 V19.3.0 (2025-12) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Requirements for support of radio resource management (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	171

1	Scope	173

2	References	173

3	Definitions, symbols and abbreviations	175

3.1	Definitions	175

3.2	Symbols	176

3.3	Abbreviations	177

3.4	Test tolerances	181

3.5	Frequency bands grouping	181

3.5.1	Introduction	181

3.5.2	NR operating bands in FR1	181

3.5.2A	NR operating bands for satellite access in FR1	148

3.5.3	NR operating bands in FR2	148

3.6	Applicability of requirements in this specification version	149

3.6.1	RRC connected state requirements in DRX	149

3.6.2	Number of serving carriers	150

3.6.2.1	Number of serving carriers for SA	150

3.6.2.2	Number of serving carriers for EN-DC	150

3.6.2.3	Number of serving carriers for NE-DC	150

3.6.2.4	Number of serving carriers for NR-DC	150

3.6.3	Applicability for intra-band FR2	150

3.6.4	Applicability for FR2 UE power classes	150

3.6.5	Applicability for SDL bands	151

3.6.6	Applicability of requirements for NGEN-DC operation	151

3.6.7	Applicability of QCL	151

3.6.9	Applicability of requirements for scheduling availability	152

3.6.10	Applicability of requirements for measurement restrictions	152

3.6.11	Applicability of requirements for Redcap UEs	152

3.6.11.1	RRC connected state requirements in DRX	152

3.6.11.2	Applicability for FR2 Redcap UE power classes	152

3.6.11.3	Applicability of QCL	152

3.6.12	Applicability of requirements for Satellite Access	152

3.6.13	Applicability of requirements for FR2	152

3.6.14	Applicability of requirements for FR2 Power Class 6	153

3.6.15	Applicability of requirements for per-FR gap	153

3.6.16	Applicability of requirements for ATG	153

3.6.17	Applicability of requirements for MUSIM gaps	153

3.6.18	Applicability of requirements for a UE operating on a cell with less than 5 MHz BW	153

3.6.19	Applicability of requirements for multi-Rx operation in FR2-1	153

3.6.20 Applicability of requirements for RedCap UE with satellite access	153

3.6.21	Applicability of requirements for UE supporting L3 fast beam sweeping operation in FR2-1	154

3.6.22	Applicability of requirements for UE with LP-WUR	154

3.6.23	Applicability of requirements for SBFD	154

4	SA: RRC_IDLE state mobility	154

4.1	Cell Selection	154

4.2	Cell Re-selection	155

4.2.1	Introduction	155

4.2.2	Requirements	155

4.2.2.1	UE measurement capability	155

4.2.2.2	Measurement and evaluation of serving cell	155

4.2.2.3	Measurements of intra-frequency NR cells	157

4.2.2.4	Measurements of inter-frequency NR cells	161

4.2.2.5	Measurements of inter-RAT E-UTRAN cells	166

4.2.2.6	Maximum interruption in paging reception	168

4.2.2.7	General requirements	169

4.2.2.8	Minimum requirement at transitions	169

4.2.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	170

4.2.2.9.1	Introduction	170

4.2.2.9.2	Measurements for UE fulfilling low mobility criterion	170

4.2.2.9.3	Measurements for UE fulfilling not-at-cell edge criterion	172

4.2.2.9.4	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	174

4.2.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	175

4.2.2.10.1	Introduction	175

4.2.2.10.2	Measurements for UE fulfilling low mobility criterion	175

4.2.2.10.3	Measurements for UE fulfilling not-at-cell edge criterion	177

4.2.2.10.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	180

4.2.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	180

4.2.2.11.1	Introduction	180

4.2.2.11.2	Measurements for UE fulfilling low mobility criterion	181

4.2.2.11.3	Measurements for UE fulfilling with not-at-cell edge criterion	182

4.2.2.11.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	184

4.2.2.12	 Measurements of inter-frequency NR cells with NTN carrier	184

4.2A	Cell Re-selection when subject to CCA	187

4.2A.1	Introduction	187

4.2A.2	Requirements	187

4.2A.2.1	UE measurement capability	187

4.2A.2.2	Measurement and evaluation when subject to CCA on the serving cell	188

4.2A.2.3	Measurements of intra-frequency NR cells when subject to CCA on the serving cell and target cell	189

4.2A.2.4	Measurements of inter-frequency NR cells when subject to CCA on the target cell	190

4.2A.2.5	Measurements of inter-RAT E-UTRAN cells when subject to CCA on the serving cell	192

4.2A.2.6	Maximum interruption in paging reception when subject to CCA on the target cell	192

4.2A.2.7	General requirements	192

4.2B	Cell Re-selection for RedCap	193

4.2B.1	Introduction	193

4.2B.2	Requirements	193

4.2B.2.1	UE measurement capability for RedCap	193

4.2B.2.1.1	UE measurement capability for 1 Rx RedCap	193

4.2B.2.1.2	UE measurement capability for 2 Rx RedCap	193

4.2B.2.2	Measurement and evaluation of serving cell for RedCap UE	193

4.2B.2.3	Measurements of intra-frequency NR cells for RedCap UE	195

4.2B.2.4	Measurements of inter-frequency NR cells for RedCap UE	197

4.2B.2.5	Measurements of inter-RAT E-UTRAN cells for RedCap UE	200

4.2B.2.6	Maximum interruption in paging reception for RedCap	202

4.2B.2.7	General requirements for RedCap	202

4.2B.2.8	Minimum requirement at transitions	202

4.2B.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion for RedCap	203

4.2B.2.9.1	Introduction	203

4.2B.2.9.2	Measurements for UE fulfilling stationary criterion	204

4.2B.2.9.3	Measurements for a UE fulfilling not-at-cell edge while stationary criterion	206

4.2B.2.9.3A	Measurements for a UE fulfilling stationary and not-at-cell-edge criteria	207

4.2B.2.9.4	Measurements for a UE fulfilling low mobility and stationary criteria	207

4.2B.2.9.5	Measurements for a UE fulfilling low mobility and not-at-cell-edge while stationary criteria	207

4.2B.2.9.6	Measurements for a UE fulfilling not-at-cell edge and not-at-cell edge while stationary criteria	208

4.2B.2.9.7	Measurements for a UE fulfilling low mobility and not-at-cell edge criteria and not-at-cell-edge while stationary criteria	208

4.2B.2.9.8	Measurements for a UE fulfilling low mobility, not-at-cell edge and stationary criterion	208

4.2B.2.9.9	Measurements for UE fulfilling low mobility criterion	208

4.2B.2.9.10	Measurements for UE fulfilling not-at-cell edge criterion	211

4.2B.2.9.11	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	213

4.2B.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	214

4.2B.2.10.1	Introduction	214

4.2B.2.10.2	Measurements for UE fulfilling stationary criterion	214

4.2B.2.10.3	Measurements for a UE fulfilling not-at-cell edge while stationary  criterion	216

4.2B.2.10.3A	Measurements for a UE fulfilling stationary and not-at-cell-edge criterion	217

4.2B.2.10.4	Measurements for a UE fulfilling low mobility and stationary criteria	217

4.2B.2.10.5	Measurements for a UE fulfilling low mobility and not-at-cell-edge while stationary criteria	217

4.2B.2.10.6	Measurements for a UE fulfilling not-at-cell edge and not-at-cell edge while stationary criteria	218

4.2B.2.10.7	Measurements for a UE fulfilling low mobility and not-at-cell edge criteria and not-at-cell-edge while stationary criteria	218

4.2B.2.10.8	Measurements for a UE fulfilling low mobility, not-at-cell edge and stationary  criteria	218

4.2B.2.10.9	Measurements for UE fulfilling low mobility criterion	218

4.2B.2.10.10	Measurements for UE fulfilling not-at-cell edge criterion	221

4.2B.2.10.11	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	223

4.2B.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	223

4.2B.2.11.1	Introduction	223

4.2B.2.11.2	Measurements for UE fulfilling stationary criterion	224

4.2B.2.11.3	Measurements for a UE fulfilling not-at-cell edge while stationary criterion	225

4.2B.2.11.3A	Measurements for a UE fulfilling stationary and not-at-cell-edge criterion	225

4.2B.2.11.4	Measurements for a UE fulfilling low mobility and stationary criteria	226

4.2B.2.11.5	Measurements for a UE fulfilling low mobility and not-at-cell-edge while stationary  criteria	226

4.2B.2.11.6	Measurements for a UE fulfilling not-at-cell edge and not-at-cell edge while stationary criteria	226

4.2B.2.11.7	Measurements for a UE fulfilling low mobility and not-at-cell edge criteria and not-at-cell-edge while stationary criteria	226

4.2B.2.11.8	Measurements for a UE fulfilling low mobility, not-at-cell edge and stationary  criteria	227

4.2B.2.11.9	Measurements for UE fulfilling low mobility criterion	227

4.2B.2.11.10	Measurements for UE fulfilling with not-at-cell edge criterion	228

4.2B.2.11.11	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	229

4.2C	Cell Re-selection for NR UE for Satellite Access	230

4.2C.1	Introduction	230

4.2C.2	Requirements	230

4.2C.2.1	UE measurement capability	230

4.2C.2.2	Measurement and evaluation of serving cell	230

4.2C.2.3	Measurements of intra-frequency NR cells	232

4.2C.2.4	Measurements of inter-frequency NR cells	234

4.2C.2.5	Maximum interruption in paging reception	237

4.2C.2.6	Minimum requirement at transitions	238

4.2C.2.7	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	238

4.2C.2.8	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	238

4.2C.2.9	General requirements	238

4.2C.2.10	Measurements of inter-frequency NR cells with TN carrier	238

4.2C.2.11	Measurements of inter-RAT E-UTRAN cells with TN carrier	241

4.2C.3	Void	243

4.2C.4	Void	243

4.2D	Cell Re-selection for ATG	243

4.2D.1	Introduction	243

4.2D.2	Requirements	243

4.2D.2.1	UE measurement capability	243

4.2D.2.2	Measurement and evaluation of serving cell	243

4.2D.2.3	Measurements of intra-frequency NR cells	244

4.2D.2.4	Measurements of inter-frequency NR cells	245

4.2D.2.5	Maximum interruption in paging reception	247

4.2D.2.6	General requirements	247

4.2E	Cell Re-selection for NR RedCap UE with Satellite Access	247

4.2E.1	Introduction	247

4.2E.2	Requirements for RedCap UE with Satellite Access	248

4.2E.2.1	UE measurement capability for RedCap with Satellite Access	248

4.2E.2.1.1	UE measurement capability for 1Rx RedCap UEs	248

4.2E.2.1.2	UE measurement capability for 2Rx RedCap UEs	248

4.2E.2.2	Measurement and evaluation of serving cell for RedCap UEs	248

4.2E.2.3	Measurements of intra-frequency NR cells for RedCap UE	249

4.2E.2.4	Measurements of inter-frequency NR cells for RedCap UE	252

4.2E.2.5	Maximum interruption in paging reception	255

4.2E.2.6	Minimum requirement at transitions for RedCap UE	255

4.2E.2.7	Measurements of intra-frequency NR cells for RedCap UE configured with relaxed measurement criterion	255

4.2E.2.7.1	Introduction	255

4.2E.2.7.2	Measurements for UE fulfilling low mobility criterion	255

4.2E.2.7.3	Measurements for UE fulfilling not-at-cell edge criterion	256

4.2E.2.7.4	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	256

4.2E.2.8	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	256

4.2E.2.8.1	Introduction	256

4.2E.2.8.2	Measurements for UE fulfilling low mobility criterion	257

4.2E.2.8.3	Measurements for UE fulfilling not-at-cell edge criterion	257

4.2E.2.8.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	257

4.2E.2.9	General requirements	258

4.2E.2.10	Measurements of inter-frequency NR cells with TN carrier	258

4.2E.2.11	Measurements of inter-RAT E-UTRAN cells with TN carrier	261

4.3	Minimization of Drive Tests (MDT)	263

4.3.1	Introduction	263

4.3.2	Measurement Requirements	263

4.3.3	Requirements for Relative Time Stamp Accuracy	263

4.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	264

4.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	264

4.3C	Minimization of Drive Tests (MDT) for Satellite Access	264

4.3C.1	Introduction	264

4.3C.2	Measurement Requirements	264

4.3C.3	Requirements for Relative Time Stamp Accuracy	265

4.3C.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	265

4.3C.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	265

4.3D	Minimization of Drive Tests (MDT) for NR RedCap UE with Satellite Access	266

4.3D.1	Introduction	266

4.3D.2	Measurement Requirements	266

4.3D.3	Requirements for Relative Time Stamp Accuracy	266

4.3D.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	266

4.3D.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	266

4.4	Idle Mode CA/DC Measurements	266

4.4.1	Introduction	266

4.4.2	Measurement Requirements	266

4.4.2.1	Detected cell requirement during state transition and Idle mode	267

4.4.2.2	Measurements of inter-frequency CA/DC candidate cells	267

4.4.2.3	Measurements on serving cell	268

4.4.2.4	Measurements of E-UTRAN inter-RAT DC candidate cells	268

4.5	NR measurements for positioning	269

4.5.1	Introduction	269

4.5.2	RSTD measurements	269

4.5.2.1	Introduction	269

4.5.2.2	Requirements Applicability	270

4.5.2.3	Measurement Capability	270

4.5.2.4	Measurement Reporting Requirements	270

4.5.2.5	Measurements Period Requirements	270

4.5.2.6	Measurements Period Requirements with Bandwidth Aggregation	273

4.5.3	PRS-RSRP measurements	277

4.5.3.1	Introduction	277

4.5.3.2	Requirements applicability	277

4.5.3.3	Measurement Capability	277

4.5.3.4	Measurement Reporting Requirements	277

4.5.3.5	Measurement Period Requirements	277

4.5.4	PRS-RSRPP measurements	280

4.5.4.1	Introduction	280

4.5.4.2	Requirements Applicability	280

4.5.4.3	Measurement Capability	280

4.5.4.4	Measurement Reporting Requirements	280

4.5.4.5	Measurement Period Requirements	281

4.5.5	Measurement requirements for DL RSCPD reported with RSTD	281

4.5.5.1	Introduction	281

4.5.5.2	Requirements Applicability	281

4.5.5.3	Measurement Capability	281

4.5.5.4	Measurement Reporting Requirements	281

4.5.5.5	Measurements Period Requirements	282

4.5A	Reporting Delay Requirements for DL AI/ML Positioning	286

4.5A.1	Introduction	286

4.5A.2	Measurements Period Requirements	286

4.5A.3	Measurements Period Requirements with Bandwidth Aggregation	289

4.6	NR measurements for positioning for RedCap	292

4.6.1	Introduction	292

4.6.2	RSTD measurements for RedCap	292

4.6.2.1	Introduction	292

4.6.2.2	Requirements Applicability	292

4.6.2.3	Measurement Capability	293

4.6.2.4	Measurement Reporting Requirements	293

4.6.2.5	Measurements Period Requirements without RX FH	293

4.6.2.6	Measurement Period Requirements with RX FH	294

4.6.3	PRS-RSRP measurements for RedCap	296

4.6.3.1	Introduction	296

4.6.3.2	Requirements applicability	296

4.6.3.3	Measurement Capability	296

4.6.3.4	Measurement Reporting Requirements	296

4.6.3.5	Measurement Period Requirements without RX FH	296

4.6.3.6	Measurement Period Requirements with RX FH	299

4.6.4	PRS-RSRPP measurements for RedCap	301

4.6.4.1	Introduction	301

4.6.4.2	Requirements Applicability	301

4.6.4.3	Measurement Capability	301

4.6.4.4	Measurement Reporting Requirements	301

4.6.4.5	Measurement Period Requirements without RX FH	302

4.6.4.6	Measurement Period Requirements with RX FH	302

4.7	Measurement report for fast CA/DC setup	302

4.7.1	Introduction	302

4.7.2	Void	302

4.7.3	Measurement Report Requirements	302

4.8	IDLE mode measurement for LP-WUS operation	303

4.8.1	Introduction	303

4.8.2 Requirements	303

4.8.2.1 UE Measurement Capability	303

4.8.2.1.1 LP-WUR measurement capability	303

4.8.2.1.2 MR measurement capability with LP-WUR	303

4.8.2.2 LP-WUR Serving cell measurement and evaluation requirement	303

4.8.2.2.1 General description	303

4.8.2.2.2 LP-WUR measurement and evaluation requirements for PSS/SSS	304

4.8.2.2.3 LP-WUR measurement and evaluation requirements for LP-SS	305

4.8.2.3	Measurement and evaluation of serving cell by MR	305

4.8.2.3.1	Requirements for evaluation of cell selection criterion	305

4.8.2.3.2	Requirements for evaluation of LP-WUS related conditions	306

4.8.2.3A	Measurement and evaluation of serving cell by RedCap UE	306

4.8.2.3A.1	Requirements for evaluation of cell selection criterion for RedCap UE	306

4.8.2.3A.2	Requirements for evaluation of LP-WUS related conditions for RedCap UE	306

4.8.2.4 Measurements of intra-frequency NR cells for UE with LP-WUR	306

4.8.2.4A	Measurements of intra-frequency NR cells for RedCap UE with LP-WUR	307

4.8.2.5	Measurements of inter-frequency NR cells for UE with LP-WUR	308

4.8.2.5.1	Introduction	308

4.8.2.5.2	Measurements for UE with LP-WUR fulfilling relaxed measurement criterion	308

4.8.2.5.3	Measurements for UE with LP-WUR fulfilling serving cell measurement offloading criterion	309

4.8.2.5A	Measurements of inter-frequency NR cells for Redcap with LP-WUR	309

4.8.2.5A.1	Introduction	309

4.8.2.5A.2	Measurements for UE with LP-WUR fulfilling relaxed measurement criterion	309

4.8.2.5A.3	Measurements for UE with LP-WUR fulfilling serving cell measurement offloading criterion	309

4.8.2.6     Measurements of inter-RAT E-UTRAN cells for UE with LP-WUR	310

4.8.2.6.1	Introduction	310

4.8.2.6.2	Measurements for UE fulfilling relaxed measurement criteria	310

4.8.2.6.3	Measurements for UE fulfilling serving cell measurement offloading entry criteria	310

4.8.2.6A	Measurements of inter-RAT E-UTRAN cells for RedCap with LP-WUR	311

4.8.2.6A.1	Introduction	311

4.8.2.6A.2	Measurements for UE fulfilling relaxed measurement criteria	311

4.8.2.6A.3	Measurements for UE fulfilling serving cell measurement offloading entry criteria	311

5	SA: RRC_INACTIVE state mobility	312

5.1	Cell Re-selection	312

5.1.1	Introduction	312

5.1.2	Requirements	312

5.1.2.1	UE measurement capability	312

5.1.2.2	Measurement and evaluation of serving cell	312

5.1.2.3	Measurements of intra-frequency NR cells	314

5.1.2.4	Measurements of inter-frequency NR cells	316

5.1.2.5	Measurements of inter-RAT E-UTRAN cells	318

5.1.2.6	Maximum interruption in paging reception	319

5.1.2.7	General requirements	319

5.1.2.8	Measurement of inter-frequency NR cells with NTN carrier	320

5.1.2.9	Minimum requirement at transitions	320

5.1.2.10	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	320

5.1.2.11	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	321

5.1.2.12	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	322

5.1A	Cell Re-selection with CCA	322

5.1A.1	Introduction	322

5.1A.2	Requirements	323

5.1A.2.1	UE measurement capability	323

5.1A.2.2	Measurement and evaluation when CCA is used on the serving cell	323

5.1A.2.3	Measurements of intra-frequency NR cells when CCA is used on the serving cell and target cell	323

5.1A.2.4	Measurements of inter-frequency NR cells when CCA is used on the target cell	323

5.1A.2.5	Measurements of inter-RAT E-UTRAN cells when CCA is used on the serving cell	323

5.1A.2.6	Maximum interruption in paging reception when CCA is used on the target cell	323

5.1A.2.7	General requirements	323

5.1B	Cell Re-selection for RedCap	323

5.1B.1	Introduction	323

5.1B.2	Requirements	323

5.1B.2.1	UE measurement capability	323

5.1B.2.2	Measurement and evaluation of serving cell	323

5.1B.2.3	Measurements of intra-frequency NR cells	326

5.1B.2.4	Measurements of inter-frequency NR cells	328

5.1B.2.5	Measurements of inter-RAT E-UTRAN cells	330

5.1B.2.6	Maximum interruption in paging reception	331

5.1B.2.7	General requirements	331

5.1B.2.8	Minimum requirement at transitions	331

5.1B.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	331

5.1B.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	333

5.1B.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	336

5.1C	Cell Re-selection for Satellite Access	337

5.1C.1	Introduction	337

5.1C.2	Requirements	337

5.1C.2.1	UE measurement capability	337

5.1C.2.2	Measurement and evaluation of serving cell	337

5.1C.2.3	Measurements of intra-frequency NR cells	337

5.1C.2.4	Measurements of inter-frequency NR cells	338

5.1C.2.5	Maximum interruption in paging reception	338

5.1C.2.6	General requirements	338

5.1C.2.7	Measurements of inter-frequency NR cells with TN carrier	338

5.1C.2.8	Measurements of inter-RAT E-UTRAN cells with TN carrier	338

5.1C.3	Void	338

5.1C.4	Void	338

5.1D	Cell Re-selection for ATG	338

5.1D.1	Introduction	338

5.1D.2	Requirements	338

5.1D.2.1	UE measurement capability	338

5.1D.2.2	Measurement and evaluation of serving cell	338

5.1D.2.3	Measurements of intra-frequency NR cells	338

5.1D.2.4	Measurements of inter-frequency NR cells	338

5.1D.2.5	Maximum interruption in paging reception	339

5.1D.2.6	General requirements	339

5.1E	Cell Re-selection for RedCap UE with Satellite Access	339

5.1E.1	Introduction	339

5.1E.2	Requirements	339

5.1E.2.1	UE measurement capability	339

5.1E.2.2	Measurement and evaluation of serving cell	339

5.1E.2.3	Measurements of intra-frequency NR cells	340

5.1E.2.4	Measurements of inter-frequency NR cells	341

5.1E.2.5	Maximum interruption in paging reception	341

5.1E.2.6	General requirements	341

5.1E.2.7	Minimum requirement at transitions	341

5.1E.2.8	Measurements of inter-frequency NR cells with TN carrier	341

5.1E.2.9	Measurements of inter-RAT E-UTRAN cells with TN carrier	342

5.2	Void	342

5.2B	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap	342

5.2B.1	Introduction	342

5.2B.2	Requirements on UE synchronization for small data transmissions for RedCap	342

5.2B.2.1	Void	342

5.2B.3	TA validation requirements for RedCap	342

5.2B.3.1	Void	343

5.2B.3.2	Void	343

5.2B.4	Scheduling restriction	343

5.2B.5	Applicability conditions for CG-SDT for RedCap	343

5.3	Minimization of Drive Tests (MDT)	344

5.3.1	Introduction	344

5.3.2	Measurement Requirements	344

5.3.3	Requirements for Relative Time Stamp Accuracy	344

5.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	344

5.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	344

5.3.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	344

5.3C	Minimization of Drive Tests (MDT) for Satellite Access	345

5.3C.1	Introduction	345

5.3C.2	Measurement Requirements	345

5.3C.3	Requirements for Relative Time Stamp Accuracy	345

5.3C.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	345

5.3C.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	345

5.3C.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	345

5.3D	Minimization of Drive Tests (MDT) for NR RedCap UE with Satellite Access	346

5.3D.1	Introduction	346

5.3D.2	Measurement Requirements	346

5.3D.3	Requirements for Relative Time Stamp Accuracy	346

5.3D.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	346

5.3D.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	346

5.3D.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	346

5.4	Inactive Mode CA/DC Measurements	347

5.4.1	Introduction	347

5.4.2	Measurement Requirements	347

5.4.2.1	Detected cell requirement during state transition and inactive mode	347

5.4.2.2	Measurements of inter-frequency CA/DC candidate cells	347

5.4.2.3	Measurements on serving cell	347

5.4.2.4	Measurements on E-UTRAN inter-RAT DC candidate cells	347

5.5	Configured Grant based Small Data Transmissions (CG-SDT)	347

5.5.1	Introduction	347

5.5.2	Requirements on UE synchronization for small data transmissions	347

5.5.3	TA validation requirements	347

5.5.4	Scheduling restriction	349

5.5.4.1	Scheduling availability of UE performing measurements in TDD bands on FR1	349

5.5.4.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	349

5.5.4.3	Scheduling availability of UE performing measurements on FR2	349

5.5.5	Applicability conditions for SDT	350

The UE is allowed to delay the reception of PRS resources on the positioning frequency layer until the SDT session is completed if the measurement using PRS resource overlaps with the SDT resources.	350

5.5D	Configured Grant based Small Data Transmissions (CG-SDT) for ATG	350

5.5E	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap UEs with NTN	350

5.5E.1	Introduction	350

5.5E.2	Requirements on UE synchronization for small data transmissions	350

5.5E.3	TA validation requirements	350

5.5E.4	Scheduling restriction	351

5.5E.5	Applicability conditions for SDT	351

5.6	NR measurements for positioning	351

5.6.1	Introduction	351

5.6.1A	Cell re-selection for positioning	352

5.6.1A.1	Measurement and evaluation of serving cell	352

5.6.1A.2	Measurements of intra-frequency NR cells	353

5.6.2	RSTD measurements	354

5.6.2.1	Introduction	354

5.6.2.2	Requirements Applicability	354

5.6.2.3	Measurement Capability	354

5.6.2.5	Measurements Period Requirements	355

5.6.2.6	Measurements Period Requirements with Bandwidth Aggregation	358

5.6.3	PRS-RSRP measurements	361

5.6.3.1	Introduction	361

5.6.3.2	Requirements applicability	361

5.6.3.3	Measurement Capability	361

5.6.3.4	Measurement Reporting Requirements	361

5.6.3.5	Measurement Period Requirements	362

5.6.4	UE Rx-Tx time difference measurements	365

5.6.4.1	Introduction	365

5.6.4.2	Requirements Applicability	365

5.6.4.3	Measurement Capability	365

5.6.4.4	Measurement Reporting Requirements	365

5.6.4.5	Measurement Period Requirements	365

5.6.4.6	Measurement Period Requirements with Bandwidth Aggregation	369

5.6.5	PRS-RSRPP measurements	372

5.6.5.1	Introduction	372

5.6.5.2	Requirements Applicability	372

5.6.5.3	Measurement Capability	372

5.6.5.4	Measurement Reporting Requirements	372

5.6.5.5	Measurement Period Requirements	373

5.6.6	TA validation requirements for positioning	373

5.6.6.1	Introduction	373

5.6.6.2	TA validation requirements	373

5.6.6.3	TA validation requirements when configured with validity area	374

5.6.7	Measurement requirements for DL RSCPD reported with RSTD	375

5.6.7.1	Introduction	375

5.6.7.2	Requirements Applicability	375

5.6.7.3	Measurement Capability	375

5.6.7.4	Measurement Reporting Requirements	375

5.6.7.5	Measurements Period Requirements	376

5.6.8	Measurement requirements for DL RSCP reported with UE Rx-Tx time difference	378

5.6.8.1	Introduction	378

5.6.8.2	Requirements Applicability	378

5.6.8.3	Measurement Capability	378

5.6.8.4	Measurement Reporting Requirements	378

5.6.8.5	Measurement Period Requirements	379

5.6A	NR measurements for positioning for RedCap	381

5.6A.1	Introduction	381

5.6A.2	Cell re-selection for positioning	382

5.6A.2.1	Measurement and evaluation of serving cell	382

5.6A.2.2	Measurements of intra-frequency NR cells	383

5.6A.3	TA validation requirements for positioning SRS	384

5.6A.3.1	Introduction	384

5.6A.3.2	TA validation requirements	384

5.6A.3.3	TA validation requirements when configured with validity area	384

5.6A.4	RSTD measurements for RedCap	384

5.6A.4.1	 Introduction	384

5.6A.4.2	Requirements applicability	384

5.6A.4.3	Measurement Capability	385

5.6A.4.4	Measurement Reporting Requirements	385

5.6A.4.5	Measurement Period Requirement without RX FH	385

5.6A.4.6	Measurement Period Requirement with RX FH	389

5.6A.5	PRS-RSRP measurements for RedCap	390

5.6A.5.1	Introduction	390

5.6A.5.2	Requirements applicability	391

5.6A.5.3	Measurement Capability	391

5.6A.5.4	Measurement Reporting Requirements	391

5.6A.5.5	Measurement Period Requirements without RX FH	392

5.6A.5.6	Measurement Period Requirement with RX FH	394

5.6A.6	UE Rx-Tx time difference measurements for RedCap	396

5.6A.6.1	Introduction	396

5.6A.6.2	Requirements Applicability	396

5.6A.6.3	Measurement Capability	397

5.6A.6.4	Measurement Reporting Requirements	397

5.6A.6.5	Measurement Period Requirements without RX FH	397

5.6A.6.6	Measurement Period Requirements with RX FH	398

5.6A.7	PRS-RSRPP measurements for RedCap	400

5.6A.7.1	 Introduction	400

5.6A.7.2	Requirements applicability	400

5.6A.7.3	Measurement Capability	400

5.6A.7.4	Measurement Reporting Requirements	400

5.6A.7.5	Measurement Period Requirements without FH	401

5.6A.7.6	Measurement period requirement with FH	401

5.6B	Reporting Delay Requirements for DL AI/ML Positioning	401

5.6B.1	Introduction	401

5.6B.2	Measurements Period Requirements	402

5.6B.3	 Measurements Period Requirements with Bandwidth Aggregation	405

5.7	Random access based Small Data Transmissions (RA-SDT)	408

5.7.1	Introduction	408

5.7.2	Requirements for small data transmissions based on 2-step RA	408

5.7.3	Requirements for small data transmissions based on 4-step RA	408

5.7.4	Applicability conditions for SDT	408

5.7B	Random access based Small Data Transmissions (RA-SDT) for RedCap	408

5.7B.1	Introduction	408

5.7B.2	Requirements for small data transmissions based on 2-step RA	408

5.7B.3	Requirements for small data transmissions based on 4-step RA	408

5.7B.4	Applicability conditions for RA-SDT for RedCap	408

5.7D	Random access based Small Data Transmissions (RA-SDT) for ATG	409

5.7E	Random access based Small Data Transmissions (RA-SDT) for RedCap UEs with NTN	409

5.7E.1	Introduction	409

5.7E.2	Requirements for small data transmissions based on 2-step RA	409

5.7E.3	Requirements for small data transmissions based on 4-step RA	409

5.7E.4	Applicability conditions for RA-SDT	409

5.8	Measurement report for fast CA/DC setup	409

5.8.1	Introduction	409

5.8.2	Void	410

5.8.3	Measurement Report Requirements	410

5.9	INACTIVE mode measurement for LP-WUS operation	410

5.9.1	Introduction	410

5.9.2 Requirements	410

5.9.2.1 UE measurement capability	410

5.9.2.1.1 LP-WUR measurement capability	410

5.9.2.1.2 MR measurement capability with LP-WUR	410

5.9.2.2	LP-WUR serving cell measurement and evaluation requirements	410

5.9.2.3	Measurement and evaluation of serving cell by MR	410

5.9.2.3A	 Measurement and evaluation of serving cell by Redcap	410

The requirements in clause 4.8.2.3A shall apply.5.9.2.4	Measurements of intra-frequency NR cells for UE with LP-WUR	410

5.9.2.4A	Measurements of intra-frequency NR cells for RedCap UE with LP-WUR	410

5.9.2.5	Measurements of inter-frequency NR cells for UE with LP-WUR	410

5.9.2.5A	Measurements of inter-frequency NR cells for Redcap with LP-WUR	411

5.9.2.6	Measurements of inter-RAT E-UTRAN cells for UE with LP-WUR	411

5.9.2.6A	Measurements of inter-RAT E-UTRAN cells for Redcap with LP-WUR	411

6	RRC_CONNECTED state mobility	411

6.1	Handover	411

6.1.1	NR Handover	411

6.1.1.1	Introduction	411

6.1.1.2	NR FR1 - NR FR1 Handover	411

6.1.1.2.1	Handover delay	411

6.1.1.2.2	Interruption time	412

6.1.1.3	NR FR2- NR FR1 Handover	413

6.1.1.3.1	Handover delay	413

6.1.1.3.2	Interruption time	413

6.1.1.4	NR FR2- NR FR2 Handover	414

6.1.1.4.1	Handover delay	414

6.1.1.4.2	Interruption time	414

6.1.1.5	NR FR1- NR FR2 Handover	415

6.1.1.5.1	Handover delay	415

6.1.1.5.2	Interruption time	416

6.1.2	NR Handover to other RATs	417

6.1.2.1	NR – E-UTRAN Handover	417

6.1.2.1.1	Introduction	417

6.1.2.1.2	Handover delay	417

6.1.2.1.3	Interruption time	417

6.1.2.2	NR – UTRAN Handover	418

6.1.2.2.1	Introduction	418

6.1.2.2.2	Handover delay	418

6.1.2.2.3	Interruption time	418

6.1.3	NR DAPS Handover	419

6.1.3.1	Introduction	419

6.1.3.2	NR FR1 - NR FR1 DAPS Handover	419

6.1.3.2.1	DAPS handover delay	419

6.1.3.2.2	Interruption time	420

6.1.3.3	NR FR2- NR FR1 DAPS Handover	421

6.1.3.3.1	DAPS handover delay	422

6.1.3.3.2	Interruption time	422

6.1.3.4	NR FR1- NR FR2 DAPS Handover	422

6.1.3.4.1	DAPS handover delay	423

6.1.3.4.2	Interruption time	423

6.1.4	NR Conditional Handover	424

6.1.4.1	Introduction	424

6.1.4.2	NR FR1 – NR FR1 conditional handover	424

6.1.4.2.2	Measurement time	424

6.1.4.3	NR FR2 – NR FR1 conditional handover	426

6.1.4.4	NR FR2 – NR FR2 conditional handover	426

6.1.4.4.1	Handover delay	426

6.1.4.4.2	Measurement time	427

6.1.4.4.3	Preparation time	428

6.1.4.4.4	Interruption time	428

6.1.4.5	NR FR1 – NR FR2 conditional handover	428

6.1.5	NR Handover with PSCell	428

6.1.5.1	Introduction	428

6.1.5.2	Handover with PSCell from NR SA to EN-DC	429

6.1.5.2.1	Interruption time for inter-RAT HO from NR to E-UTRAN	429

6.1.5.2.2	PSCell addition in HO with PSCell for NR SA to EN-DC	429

6.1.5.3	HO with PSCell from NE-DC to NE-DC	430

6.1.5.3.1	Handover delay	430

6.1.5.3.2	HO with PSCell - PCell Interruption time	430

6.1.5.3.3	PSCell addition/change in NE-DC to NE-DC HO with PSCell	430

6.1.5.4	HO with PSCell from NR-DC to NR-DC	431

6.1.5.5	Handover with PSCell from NR SA to EN-DC with PSCell using CCA	432

6.1.5.5.1	Introduction	432

6.1.5.5.2	NR SA to EN-DC HO with PSCell- NR to E-UTRA HO Interruption time	432

6.1.5.5.3	NR SA to EN-DC HO with PSCell - NR PSCell Addition Delay requirements	433

6.1.6	NR Conditional Handover including target MCG and target SCG	434

6.1.6.1	Conditional handover including target MCG in FR1 and target SCG in FR1 in NR-DC	434

6.1.6.1.1	CHO with PSCell – PCell Interruption time	434

6.1.6.1.2	CHO with PSCell – PSCell change delay	435

6.1.6.2	Conditional handover including target MCG in FR1 and target SCG in FR2 in NR-DC	435

6.1.6.2.2	CHO with PSCell – PSCell change delay	436

6.1.7	NR Conditional Handover including target MCG and candidate SCG	437

6.1.7.1	Conditional handover including target MCG and candidate SCG for CPC in FR1 NR-DC	437

6.1.7.1.1	PCell conditional handover delay	438

6.1.7.1.2	PSCell conditional change delay	439

6.1.7.2	Conditional handover including target MCG in FR1 and Candidate SCG for CPC in FR2 in NR-DC	440

6.1.7.2.1	PCell handover delay	440

6.1.7.2.2	PSCell conditional change delay	441

6.1A	Void	443

6.1A.1	Void	443

6.1A.1.1	Void	443

6.1A.1.2	Void	443

6.1A.1.2.1	Void	443

6.1A.1.2.2	Void	443

6.1B	Handover to target cell using CCA	443

6.1B.1	NR Handover	443

6.1B.1.1	Introduction	443

6.1B.1.2	NR FR1 - NR FR1 Handover	443

6.1B.1.2.1	Handover delay	443

6.1B.1.2.2	Interruption time	443

6.1B.1.3	NR FR2-2 NR FR2-2 Handover	444

6.1B.1.3.1	Handover delay	444

6.1B.1.3.2	Interruption time	444

6.1B.1.4	NR FR1- NR FR2-2 Handover	445

6.1B.1.4.1	Handover delay	445

6.1B.1.4.2	Interruption time	446

6.1C	Handover for SAN	447

6.1C.1	NR SAN Handover	447

6.1C.1.1	Introduction	447

6.1C.1.2	NR SAN FR1 – NR SAN FR1 Handover	447

6.1C.1.2.1	Handover delay	447

6.1C.1.2.2	Interruption time	447

6.1C.1.3	NR SAN FR2-NTN – NR SAN FR2-NTN Handover	448

6.1C.1.3.1	Handover delay	448

6.1C.1.3.2	Interruption time	449

6.1C.2	NR SAN Conditional Handover	449

6.1C.2.1	Introduction	449

6.1C.2.2	NR SAN FR1 – NR SAN FR1 conditional handover	450

6.1C.2.2.1	Handover delay	450

6.1C.2.2.2	Measurement time	450

6.1C.2.2.3	Preparation time	452

6.1C.2.2.4	Interruption time	452

6.1C.2.3	NR SAN FR1 – NR SAN FR1 conditional handover without L3 measurement criteria	452

6.1C.2.3.1	Handover delay	452

6.1C.2.3.2	Preparation time	453

6.1C.2.3.3	Interruption time	453

6.1C.2.4	NR SAN FR2-NTN – NR SAN FR2-NTN conditional handover	454

6.1C.3	NR SAN Satellite switching with re-synchronization	454

6.1C.3.1	Introduction	454

6.1C.3.2	NR SAN FR1 – NR SAN FR1 Satellite switching with re-synchronization	454

6.1C.3.2.1	Satellite switching delay	454

6.1C.3.2.2	Interruption time for hard satellite switch with re-sync	454

6.1C.3.3	NR SAN FR2 – NR SAN FR2 Satellite switching with re-synchronization	456

6.1C.3.3.1	Satellite switching delay	456

6.1C.3.3.2	Interruption time for hard satellite switch with re-sync	456

6.1C.3.3.3	Satellite switch delay for soft satellite switch with re-sync	456

6.1D	Handover for RedCap	457

6.1D.1	NR Handover	457

6.1D.1.1	Introduction	457

6.1D.1.2	NR FR1 - NR FR1 Handover	457

6.1D.1.2.1	Handover delay	458

6.1D.1.2.2	Interruption time	458

6.1D.1.3	NR FR2- NR FR2 Handover	459

6.1D.1.3.1	Handover delay	459

6.1D.1.3.2	Interruption time	459

6.1D.2	NR Handover to other RATs	461

6.1D.2.1	NR – E-UTRAN Handover	461

6.1E	Handover for ATG	461

6.1E.1	NR Handover	461

6.1E.1.1	Introduction	461

6.1E.1.2	NR FR1 - NR FR1 Handover	461

6.1E.1.2.1	Handover delay	461

6.1E.1.2.2	Interruption time	461

6.1E.2	NR Conditional Handover	462

6.1E.2.1	Introduction	462

6.1E.2.2	NR FR1 – NR FR1 conditional handover	462

6.1E.2.2.1	Handover delay	462

6.1E.2.2.2	Measurement time	463

6.1E.2.2.3	Preparation time	463

6.1E.2.2.4	Interruption time	463

6.1F	Handover for RedCap UE with satellite access	464

6.1F.1	NR SAN Handover	464

6.1F.1.1	Introduction	464

6.1F.1.2	NR SAN FR1 – NR SAN FR1 Handover	464

6.1F.1.2.1	Handover delay	464

6.1F.1.2.2	Interruption time	464

6.1F.2	NR SAN Conditional Handover	464

6.1F.2.1	Introduction	464

6.1F.2.2	NR SAN FR1 – NR SAN FR1 conditional handover	465

6.1F.2.2.1	Handover delay	465

6.1F.2.2.2	Measurement time	465

6.1F.2.2.3	Preparation time	465

6.1F.2.2.4	Interruption time	465

6.1F.2.3	NR SAN FR1 – NR SAN FR1 conditional handover without L3 measurement criteria	465

6.1F.2.3.1	Handover delay	465

6.1F.2.3.2	Preparation time	465

6.1F.2.3.3	Interruption time	465

6.1F.3	NR SAN Satellite switching with re-synchronization	466

6.1F.3.1	Introduction	466

6.1F.3.2	NR SAN FR1 – NR SAN FR1 Satellite switching with re-synchronization	466

6.1F.3.2.1	Satellite switching delay	466

6.1F.3.2.2	Interruption time for hard satellite switch with re-sync	466

6.1F.3.2.3	Satellite switch delay for soft satellite switch with re-sync	466

6.2	RRC Connection Mobility Control	467

6.2.1	SA: RRC Re-establishment	467

6.2.1.1	Introduction	467

6.2.1.2	Requirements	467

6.2.1.2.1	UE Re-establishment delay requirement	467

6.2.1A	RRC Re-establishment with CCA	469

6.2.1A.1	Introduction	469

6.2.1A.2	Requirements	470

6.2.1A.2.1	UE Re-establishment with CCA delay requirement	470

6.2.1B	SA: RRC Re-establishment for RedCap	471

6.2.1B.1	Introduction	471

6.2.1B.2	Requirements	472

6.2.2	Random access	472

6.2.2.1	Introduction	472

6.2.2.2	Requirements for 4-step RA type	472

6.2.2.2.1	Contention based random access	473

6.2.2.2.1.1	Correct behaviour when transmitting Random Access Preamble	473

6.2.2.2.1.2	Correct behaviour when receiving Random Access Response	473

6.2.2.2.1.3	Correct behaviour when not receiving Random Access Response	473

6.2.2.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	473

6.2.2.2.1.5	SA: Correct behaviour when receiving a message over Temporary C-RNTI	473

6.2.2.2.1.6	Correct behaviour when contention Resolution timer expires	473

6.2.2.2.2	Non-Contention based random access	474

6.2.2.2.2.1	Correct behaviour when transmitting Random Access Preamble	474

6.2.2.2.2.2	Correct behaviour when receiving Random Access Response	474

6.2.2.2.2.3	Correct behaviour when not receiving Random Access Response	474

6.2.2.2.3	UE behaviour when configured with supplementary UL	475

6.2.2.3	Requirements for 2-step RA type	475

6.2.2.3.1	Contention based random access	475

6.2.2.3.1.1	Correct behaviour when transmitting MsgA	475

6.2.2.3.1.2	Correct behaviour when receiving MsgB	475

6.2.2.3.1.3	Correct behaviour when not receiving MsgB	476

6.2.2.3.2	Non-Contention based random access	476

6.2.2.3.2.1	Correct behaviour when transmitting MsgA	476

6.2.2.3.2.2	Correct behaviour when receiving MsgB	476

6.2.2.3.2.3	Correct behaviour when not receiving MsgB	476

6.2.2.3.3	UE behaviour when configured with supplementary UL	476

6.2.2A	Random access when CCA is used on target frequency	477

6.2.2A.1	Introduction	477

6.2.2A.2	Requirements for 4-step RA type	477

6.2.2A.2.1	Contention based random access	477

6.2.2A.2.1.1	Correct behaviour when transmitting Random Access Preamble	477

6.2.2A.2.1.2	Correct behaviour when receiving Random Access Response	478

6.2.2A.2.1.3	Correct behaviour when not receiving Random Access Response	478

6.2.2A.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	478

6.2.2A.2.1.6	Correct behaviour when contention Resolution timer expires	478

6.2.2A.2.2	Non-Contention based random access	478

6.2.2A.2.2.1	Correct behaviour when transmitting Random Access Preamble	478

6.2.2A.2.2.2	Correct behaviour when receiving Random Access Response	479

6.2.2A.2.2.3	Correct behaviour when not receiving Random Access Response	479

6.2.2A.3	Requirements for 2-step RA type	479

6.2.2A.3.1	Contention based random access	479

6.2.2A.3.1.1	Correct behaviour when transmitting MsgA	479

6.2.2A.3.1.2	Correct behaviour when receiving MsgB	480

6.2.2A.3.1.3	Correct behaviour when not receiving MsgB	480

6.2.2A.3.2	Non-Contention based random access	480

6.2.2A.3.2.1	Correct behaviour when transmitting MsgA	480

6.2.2A.3.2.2	Correct behaviour when receiving MsgB	481

6.2.2A.3.2.3	Correct behaviour when not receiving MsgB	481

6.2.2B	Random access for RedCap	481

6.2.2B.1	Introduction	481

6.2.2B.2	Requirements	482

6.2.2C	PDCCH ordered Random Access for LTM	482

6.2.2C.1	Introduction	482

6.2.2C.2	PDCCH ordered Random Access delay	482

6.2.3	SA: RRC Connection Release with Redirection	483

6.2.3.1	Introduction	483

6.2.3.2	Requirements	483

6.2.3.2.1	RRC connection release with redirection to NR	483

6.2.3.2.2	RRC connection release with redirection to E-UTRAN	484

6.2.3.2.3	RRC connection release with redirection to NR carrier subject to CCA	485

6.2.3A	SA: RRC Connection Release with Redirection for RedCap	486

6.2.3A.1	Introduction	486

6.2.3A.2	Requirements	486

6.2.3A.2.1	RRC connection release with redirection to NR	486

6.2.3A.2.2	RRC connection release with redirection to E-UTRAN	486

6.2C	RRC Connection Mobility Control for Satellite Access	486

6.2C.1	SA: RRC Re-establishment for Satellite Access	486

6.2C.1.1	Introduction	486

6.2C.1.2	Requirements	487

6.2C.1.2.1	UE Re-establishment delay requirement	487

6.2C.1.2.2	UE Re-establishment delay requirement for VSAT	489

6.2C.2	Random access for satellite access	489

6.2C.2.1	Introduction	489

6.2C.2.2	Requirements for 4-step RA type	489

6.2C.2.2.1	Contention based random access	490

6.2C.2.2.1.1	Correct behaviour when transmitting Random Access Preamble	490

6.2C.2.2.1.2	Correct behaviour when receiving Random Access Response	490

6.2C.2.2.1.3	Correct behaviour when not receiving Random Access Response	490

6.2C.2.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	490

6.2C.2.2.1.5	SA: Correct behaviour when receiving a message over Temporary C-RNTI	490

6.2C.2.2.1.6	Correct behaviour when Contention Resolution Timer expires	490

6.2C.2.2.2	Non-Contention based random access	490

6.2C.2.2.2.1	Correct behaviour when transmitting Random Access Preamble	490

6.2C.2.2.2.2	Correct behaviour when receiving Random Access Response	491

6.2C.2.2.2.3	Correct behaviour when not receiving Random Access Response	491

6.2C.2.3	Requirements for 2-step RA type	491

6.2C.2.3.1	Contention based random access	492

6.2C.2.3.1.1	Correct behaviour when transmitting MsgA	492

6.2C.2.3.1.2	Correct behaviour when receiving MsgB	492

6.2C.2.3.1.3	Correct behaviour when not receiving MsgB	492

6.2C.2.3.2	Non-Contention based random access	492

6.2C.2.3.2.1	Correct behaviour when transmitting MsgA	492

6.2C.2.3.2.2	Correct behaviour when receiving MsgB	493

6.2C.2.3.2.3	Correct behaviour when not receiving MsgB	493

6.2C.3	SA: RRC Connection Release with Redirection for Satellite Access	493

6.2C.3.1	Introduction	493

6.2C.3.2	Requirements	493

6.2C.3.2.1	RRC connection release with redirection to NR	493

6.2D	RRC Connection Mobility Control for ATG	494

6.2D.1	SA: RRC Re-establishment	494

6.2D.1.1	Introduction	494

6.2D.1.2	Requirements	494

6.2D.1.2.1	UE Re-establishment delay requirement	494

6.2D.2	Random access	496

6.2D.2.1	Introduction	496

6.2D.2.2	Requirements for 4-step RA type	496

6.2D.2.3	Requirements for 2-step RA type	496

6.2D.3	SA: RRC Connection Release with Redirection	496

6.2D.3.1	Introduction	496

6.2D.3.2	Requirements	496

6.2D.3.2.1	RRC connection release with redirection to NR	496

6.2E	RRC Connection Mobility Control for RedCap UE with Satellite Access	497

6.2E.1	SA: RRC Re-establishment for RedCap UE with Satellite Access	497

6.2E.1.1	Introduction	497

6.2E.1.2	Requirements	498

6.2E.2	Random access for RedCap UE with satellite access	498

6.2E.2.1	Introduction	498

6.2E.2.2	Requirements	498

6.2E.3	SA: RRC Connection Release with Redirection for RedCap UE with Satellite Access	499

6.2E.3.1	Introduction	499

6.2E.3.2	Requirements	499

6.2E.3.2.1	RRC connection release with redirection to NR	499

6.3	L1/L2-Triggered Mobility	499

6.3.1	LTM PCell Cell Switch	499

6.3.1.1	Introduction	499

6.3.1.2	LTM Cell Switch delay	501

6.3.1.3	Interruption time	501

6.3.2	Conditional L1/L2-Triggered Mobility	503

6.3.2.1	Introduction	503

6.3.2.2	CLTM Cell Switch delay	503

6.3.2.2.1	Measurement time	503

6.3.2.2.2	CLTM RRC processing time	504

6.3.2.2.3	Interruption time	505

6.3.2.3	Subsequent CLTM Cell Switch delay	506

7	Timing	506

7.1	UE transmit timing	506

7.1.1	Introduction	506

7.1.2	Requirements	507

7.1.2.1	Gradual timing adjustment	509

7.1.2.2	Void	510

7.1.2.3	One shot large UL timing adjustment for FR2 Power Class 6 UE	510

7.1.2.4	UE transmit timing for positioning measurements	511

7.1A	UE transmit timing for RedCap	511

7.1A.1	Introduction	511

7.1A.2	Requirements	511

7.1A.2.1	Gradual timing adjustment	512

7.1A.2.2	UE transmit timing for positioning measurements	513

7.1C	UE transmit timing for Satellite Access	513

7.1C.1	Introduction	513

7.1C.2	Requirements	513

7.1C.2.1	Gradual timing adjustment	515

7.1D	UE transmit timing for ATG	515

7.1D.1	Introduction	515

7.1D.2	Requirements	515

7.1D.2.1	Gradual timing adjustment	516

7.1E	UE transmit timing for RedCap with Satellite Access	516

7.1E.1	Introduction	516

7.1E.2	Requirements	517

7.1E.2.1	Gradual timing adjustment	517

7.2	UE timer accuracy	517

7.2.1	Introduction	517

7.2.2	Requirements	517

7.2A	UE timer accuracy for RedCap	517

7.2A.1	Introduction	517

7.2A.2	Requirements	517

7.2C	UE timer accuracy for satellite access	518

7.2C.1	Introduction	518

7.2C.2	Requirements	518

7.2D	UE timer accuracy for ATG	518

7.2D.1	Introduction	518

7.2D.2	Requirements	518

7.2E	UE timer accuracy for RedCap with Satellite Access	519

7.2E.1	Introduction	519

7.2E.2	Requirements	519

7.3	Timing advance	519

7.3.1	Introduction	519

7.3.2	Requirements	519

7.3.2.1	Timing Advance adjustment delay	519

7.3.2.2	Timing Advance adjustment accuracy	519

7.3A	Timing Advance for RedCap	519

7.3A.1	Introduction	519

7.3A.2	Requirements	520

7.3A.2.1	Timing Advance adjustment delay	520

7.3A.2.2	Timing Advance adjustment accuracy	520

7.3C	Timing advance for satellite access	520

7.3C.1	Introduction	520

7.3C.2	Requirements	520

7.3C.2.1	Timing Advance adjustment delay	520

7.3C.2.2	Timing Advance adjustment accuracy	520

7.3D	Timing advance for ATG	521

7.3D.1	Introduction	521

7.3D.2	Requirements	521

7.3D.2.1	Timing Advance adjustment delay	521

7.3D.2.2	Timing Advance adjustment accuracy	521

7.3E	Timing advance for RedCap with Satellite Access	521

7.3E.1	Introduction	521

7.3E.2	Requirements	521

7.3E.2.1	Timing Advance adjustment delay	521

7.3E.2.2	Timing Advance adjustment accuracy	521

7.4	Cell phase synchronization accuracy	521

7.4.1	Definition	521

7.4.2	Minimum requirements	522

7.5	Maximum Transmission Timing Difference	522

7.5.1	Introduction	522

7.5.2	Minimum requirements for inter-band EN-DC	522

7.5.2.1	Minimum requirements for inter-band synchronous EN-DC	522

7.5.3	Minimum requirements for intra-band EN-DC	523

7.5.4	Minimum requirements for NR Carrier Aggregation	524

7.5.5	Minimum requirements for inter-band NE-DC	525

7.5.5.1	Minimum requirements for inter-band synchronous NE-DC	525

7.5.6	Minimum requirements for inter-band NR-DC	525

7.5.7	Minimum requirements for multi-TRP	526

7.6	Maximum Receive Timing Difference	526

7.6.1	Introduction	526

7.6.2	Minimum requirements for inter-band EN-DC	527

7.6.2.1	Minimum requirements for inter-band synchronous EN-DC	527

7.6.3	Minimum requirements for intra-band EN-DC	529

7.6.4	Minimum requirements for NR Carrier Aggregation	529

7.6.5	Minimum requirements for inter-band NE-DC	530

7.6.5.1	Minimum requirements for inter-band synchronous NE-DC	531

7.6.6	Minimum requirements for inter-band NR-DC	531

7.6.7	Minimum requirements for PC6 UE in FR2	532

7.6.8	Minimum requirements for Multi-TRPs	532

7.6D	Maximum Receive Timing Difference for ATG UE	533

7.6D.1	Introduction	533

7.6D.2	Minimum requirements for NR Carrier Aggregation	533

7.7	deriveSSB-IndexFromCell tolerance	533

7.7.1	Minimum requirements	533

7.7A	deriveSSB-IndexFromCell tolerance for RedCap	534

7.7A.1	Minimum requirements	534

7.7D	DeriveSSB-IndexFromCell tolerance for ATG	534

7.7D.1	Minimum requirements	534

7.8	Void	534

7.9	deriveSSB-IndexFromCellInter-r17 tolerance	534

7.9.1	Minimum requirements	534

7.9D	DeriveSSB-IndexFromCellInter-r17 tolerance for ATG	535

7.9D.1	Minimum requirements	535

8	Signalling characteristics	572

8.1	Radio Link Monitoring	572

8.1.1	Introduction	572

8.1.1.1	Introduction of Requirement on Radio Link Monitoring for UE Configured with Relaxed Measurement Criteria	573

8.1.2	Requirements for SSB based radio link monitoring	574

8.1.2.1	Introduction	574

8.1.2.2	Minimum requirement	575

8.1.2.3	Measurement restrictions for SSB based RLM	579

8.1.2.4	Minimum requirement of SSB based radio link monitoring for UE fulfilling relaxed measurement criteria	580

8.1.3	Requirements for CSI-RS based radio link monitoring	581

8.1.3.1	Introduction	581

8.1.3.2	Minimum requirement	581

8.1.3.3	Measurement restrictions for CSI-RS based RLM	586

8.1.3.4	Minimum requirement of CSI-RS based radio link monitoring for UE fulfilling relaxed measurement criteria	588

8.1.4	Minimum requirement at transitions	588

8.1.5	Minimum requirement for UE turning off the transmitter	589

8.1.6	Minimum requirement for L1 indication	589

8.1.7	Scheduling availability of UE during radio link monitoring	589

8.1.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	589

8.1.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	590

8.1.7.3	Scheduling availability of UE performing radio link monitoring on FR2	590

8.1.7.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	591

8.1.8	Minimum requirement under IDC Interference	592

8.1A	Radio Link Monitoring with CCA on Target Frequency	592

8.1A.1	Introduction	592

8.1A.2	Requirements for SSB Based Radio Link Monitoring	593

8.1A.2.1	Introduction	593

8.1A.2.2	Minimum Requirement	593

8.1A.2.3	Measurement Restrictions for SSB based RLM	596

8.1A.3	Minimum requirement at transitions	596

8.1A.4	Minimum requirement for UE turning off the transmitter	597

8.1A.5	Minimum requirement for L1 indication	597

8.1A.6	Scheduling availability of UE during radio link monitoring	597

8.1A.6.3	Scheduling availability of UE performing radio link monitoring on FR2-2	597

8.1A.6.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR-DC	598

8.1B	Radio Link Monitoring for RedCap	598

8.1B.1	Introduction	598

8.1B.2	Requirements for SSB based radio link monitoring	599

8.1B.2.1	Introduction	599

8.1B.2.2	Minimum requirement	600

8.1B.2.3	Measurement restrictions for SSB based RLM	602

8.1B.3	Requirements for CSI-RS based radio link monitoring	602

8.1B.3.1	Introduction	602

8.1B.3.2	Minimum requirement	603

8.1B.3.3	Measurement restrictions for CSI-RS based RLM	605

8.1B.4	Minimum requirement at transitions	606

8.1B.5	Minimum requirement for UE turning off the transmitter	606

8.1B.6	Minimum requirement for L1 indication	606

8.1B.7	Scheduling availability of UE during radio link monitoring	607

8.1B.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	607

8.1B.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	607

8.1B.7.3	Scheduling availability of UE performing radio link monitoring on FR2	607

8.1C	Radio Link Monitoring for Satellite Access	608

8.1C.1	Introduction	608

8.1C.2	Requirements for SSB based radio link monitoring	609

8.1C.2.1	Introduction	609

8.1C.2.2	Minimum requirement	610

8.1C.2.3	Measurement restrictions for SSB based RLM	611

8.1C.3	Requirements for CSI-RS based radio link monitoring	612

8.1C.3.1	Introduction	612

8.1C.3.2	Minimum requirement	612

8.1C.3.3	Measurement restrictions for CSI-RS based RLM	614

8.1C.4	Minimum requirement at transitions	614

8.1C.5	Minimum requirement for UE turning off the transmitter	614

8.1C.6	Minimum requirement for L1 indication	615

8.1C.7	Scheduling availability of UE during radio link monitoring	615

8.1C.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1-NTN and FR2-NTN	615

8.1C.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1-NTN and FR2-NTN	615

8.1D	Radio Link Monitoring for ATG	615

8.1D.1	Introduction	615

8.1D.2	Requirements for SSB based radio link monitoring	616

8.1D.2.1	Introduction	616

8.1D.2.2	Minimum requirement	617

8.1D.2.3	Measurement restrictions for SSB based RLM	618

8.1D.3	Requirements for CSI-RS based radio link monitoring	618

8.1D.3.1	Introduction	618

8.1D.3.2	Minimum requirement	618

8.1D.3.3	Measurement restrictions for CSI-RS based RLM	620

8.1D.4	Minimum requirement at transitions	620

8.1D.5	Minimum requirement for UE turning off the transmitter	620

8.1D.6	Minimum requirement for L1 indication	620

8.1D.7	Scheduling availability of UE during radio link monitoring	620

8.1D.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	620

8.1D.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	620

8.1E	Radio Link Monitoring for RedCap UE with Satellite Access	621

8.1E.1	Introduction	621

8.1E.2	Requirements for SSB based radio link monitoring	621

8.1E.2.1	Introduction	621

8.1E.2.2	Minimum requirement	622

8.1E.2.3	Measurement restrictions for SSB based RLM	622

8.1E.3	Requirements for CSI-RS based radio link monitoring	622

8.1E.3.1	Introduction	622

8.1E.3.2	Minimum requirement	623

8.1E.3.3	Measurement restrictions for CSI-RS based RLM	624

8.1E.4	Minimum requirement at transitions	624

8.1E.5	Minimum requirement for UE turning off the transmitter	624

8.1E.6	Minimum requirement for L1 indication	624

8.1E.7	Scheduling availability of UE during radio link monitoring	624

8.1E.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH	624

8.1E.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH	624

8.2	Interruption	624

8.2.1	EN-DC Interruption	624

8.2.1.1	Introduction	624

8.2.1.2	Requirements	625

8.2.1.2.1	Interruptions at transitions between active and non-active during DRX	625

8.2.1.2.2	Interruptions at transitions from non-DRX to DRX	626

8.2.1.2.3	Interruptions at SCell addition/release	626

8.2.1.2.4	Interruptions at SCell activation/deactivation	628

8.2.1.2.5	Interruptions during measurements on SCC	631

8.2.1.2.6	Interruptions at UL carrier RRC reconfiguration	633

8.2.1.2.7	Interruptions due to Active BWP switching Requirement	633

8.2.1.2.8	Interruptions at direct SCell activation and hibernation	634

8.2.1.2.9	Interruptions at SCell hibernation	635

8.2.1.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	635

8.2.1.2.11	Interruptions due to UE-specific CBW change	635

8.2.1.2.12	Interruptions at NR SRS carrier based switching	636

8.2.1.2.13	Interruptions at E-UTRA SRS carrier based switching	637

8.2.1.2.14	DL Interruptions at switching between two uplink carriers	638

8.2.1.2.15	Interruptions due to SCell dormancy	638

8.2.1.2.16	Interruptions when identifying CGI of an NR cell with autonomous gaps	639

8.2.1.2.17	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	639

8.2.1.2.18	Interruptions at NR SRS antenna port switching	640

8.2.1.2.19	Interruptions at fast SCell activation	641

8.2.1.2.20	Interruptions due to PUCCH SCell activation/deactivation	642

8.2.1.2.21	Interruptions at OD-SSB activation/deactivation	642

8.2.2	SA: Interruptions with Standalone NR Carrier Aggregation	643

8.2.2.1	Introduction	643

8.2.2.2	Requirements	644

8.2.2.2.1	Interruptions at SCell addition/release	644

8.2.2.2.2	Interruptions at SCell activation/deactivation	645

8.2.2.2.3	Interruptions during measurements on deactivated SCC	647

8.2.2.2.4	Interruptions at UL carrier RRC reconfiguration	649

8.2.2.2.5	Interruptions due to Active BWP switching Requirement	649

8.2.2.2.6	Interruptions at inter-frequency SFTD measurement	650

8.2.2.2.7	Interruptions at SCell activation/deactivation with multiple downlink SCells	651

8.2.2.2.8	Interruptions due to UE-specific CBW change	651

8.2.2.2.9	Interruptions at NR SRS carrier based switching	652

8.2.2.2.10	DL Interruptions at UE switching between two uplink carriers	653

8.2.2.2.10A	DL Interruptions at UE switching between two uplink carriers with two transmit antenna connectors	654

8.2.2.2.10B	DL Interruptions at UE switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	654

8.2.2.2.10C	DL Interruptions at UE switching between two uplink bands with two transmit antenna connectors	655

8.2.2.2.10D	DL Interruptions at UE switching across three or four uplink bands	655

8.2.2.2.10E	DL Interruptions at UE switching between two uplink bands with three transmit antenna connectors and maximum two transmit antenna connectors for each band	656

8.2.2.2.11	Interruptions at direct SCell activation	656

8.2.2.2.12	Interruptions due to SCell dormancy	657

8.2.2.2.12.1	Interruptions due to SCell dormancy switch	657

8.2.2.2.12.2	Interruptions due to CQI measurements during SCell dormancy	657

8.2.2.2.12.3	Interruptions due to RRM measurements during SCell dormancy	657

8.2.2.2.13	Interruptions at transitions between active and non-active during DRX	657

8.2.2.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	657

8.2.2.2.15	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	658

8.2.2.2.16	Interruptions at NR SRS antenna port switching	658

8.2.2.2.17	Interruptions at fast SCell activation	659

8.2.2.2.18	Interruptions due to PUCCH SCell activation/deactivation	660

8.2.2.2.19	Interruptions due to measurements without gap carried out by UE supporting NeedForInterruptionInfoNR	660

8.2.2.2.20	Interruptions due to PDCCH ordered RACH on target LTM cell	661

8.2.2.2.21	Interruptions at NR SRS bandwidth aggregation for positioning	661

8.2.2.2.22	Interruptions at OD-SSB activation/deactivation	664

8.2.3	NE-DC Interruptions	665

8.2.3.1	Introduction	665

8.2.3.2	Requirements	666

8.2.3.2.1	Interruptions at transitions between active and non-active during DRX	666

8.2.3.2.2	Interruptions at transitions from non-DRX to DRX	666

8.2.3.2.3	Interruptions at PSCell/SCell addition/release	666

8.2.3.2.4	Interruptions at SCell activation/deactivation	667

8.2.3.2.5	Interruptions during measurements on SCC	668

8.2.3.2.5.1	Interruptions during measurements on deactivated NR SCC	668

8.2.3.2.5.2	Interruptions during measurements on deactivated E-UTRAN SCC	668

8.2.3.2.5.3	Interruptions during CQI measurements on dormant E-UTRAN SCC	669

8.2.3.2.5.4	Interruptions during RRM measurements on dormant E-UTRAN SCC	669

8.2.3.2.6	Interruptions at UL carrier RRC reconfiguration	669

8.2.3.2.7	Interruptions due to Active BWP switching Requirement	670

8.2.3.2.8	Interruptions at direct SCell activation and hibernation	670

8.2.3.2.9	Interruptions at SCell hibernation	670

8.2.3.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	671

8.2.3.2.11	Interruptions at NR SRS carrier based switching	671

8.2.3.2.12	Interruptions at E-UTRA SRS carrier based switching	672

8.2.3.2.13	Interruptions due to SCell dormancy	673

8.2.3.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	673

8.2.3.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	674

8.2.3.2.17	Interruptions at fast SCell activation	676

8.2.3.2.18	Interruptions due to UE-specific CBW change	676

8.2.3.2.19	Interruptions due to PUCCH SCell activation/deactivation	676

8.2.3.2.20	Interruptions at OD-SSB activation/deactivation	676

8.2.4	NR-DC: Interruptions	677

8.2.4.1	Introduction	677

8.2.4.2	Requirements	678

8.2.4.2.1	Interruptions at PSCell/SCell addition/release	678

8.2.4.2.2	Interruptions at SCell activation/deactivation	679

8.2.4.2.3	Interruptions during measurements on SCC	680

8.2.4.2.4	Interruptions at UL carrier RRC reconfiguration	680

8.2.4.2.5	Interruptions due to Active BWP switching Requirement	680

8.2.4.2.6	Interruptions at transitions between active and non-active during DRX	681

8.2.4.2.7	Interruptions at transitions from non-DRX to DRX	681

8.2.4.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	681

8.2.4.2.9	Interruptions at NR SRS carrier based switching	681

8.2.4.2.10	Interruptions at direct SCell activation	683

8.2.4.2.11	Interruptions when identifying CGI of an NR cell with autonomous gaps	684

8.2.4.2.12	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	684

8.2.4.2.13	 Interruptions due to SCell dormancy	685

8.2.4.2.14	Interruptions at NR SRS antenna port switching	685

8.2.4.2.15	Interruptions at fast SCell activation	686

8.2.4.2.16	Interruptions at SCG activation/deactivation	687

8.2.4.2.17	Interruptions due to RRM measurements on deactivated SCG	687

8.2.4.2.18	Interruptions during RLM/BFD measurements on deactivated PSCell	687

8.2.4.2.19	Interruptions due to UE-specific CBW change	687

8.2.4.2.20	Interruptions due to PDCCH ordered RACH on target LTM cell	688

8.2.4.2.21	Interruptions at PSCell Cell switch	688

8.2.4.2.22	Interruptions at OD-SSB activation/deactivation	688

8.2.4.2A	Void	689

8.2.4.2A.1	Void	689

8.2.4.2A.2	Void	689

8.2.4.2A.3	Void	689

8.2D	Interruption for ATG UE	689

8.2D.1	Interruptions with Standalone NR Carrier Aggregation	689

8.2D.1.1	Introduction	689

8.2D.1.2	Requirements	690

8.2D.1.2.1	Interruptions at SCell addition/release	690

8.2D.1.2.2	Interruptions at SCell activation/deactivation	690

8.2D.1.2.3	Interruptions during measurements on deactivated SCC	691

8.2D.1.2.4	Interruptions at direct SCell activation	691

8.2D.1.2.5	Interruptions due to SCell dormancy	692

8.2D.1.2.6	Interruptions at fast SCell activation	692

8.2D.1.2.8	Interruptions due to UE-specific CBW change	693

8.2D.1.2.9	Interruptions when identifying CGI of an NR cell with autonomous gaps	694

8.2D.1.2.10	Interruptions at NR SRS antenna port switching	694

8.3	SCell Activation and Deactivation Delay	695

8.3.1	Introduction	695

8.3.2	SCell Activation Delay Requirement for Deactivated SCell	695

8.3.2A	SCell Activation Delay Requirement for Deactivated SCell based on measurement in IDLE/INACTIVE mode	703

8.3.3	SCell Deactivation Delay Requirement for Activated SCell	705

8.3.4	Direct SCell Activation at SCell addition	705

8.3.5	Direct SCell Activation at Handover	707

8.3.7	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	709

8.3.8	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	713

8.3.9	Direct SCell Activation of Multiple Downlink SCells at SCell addition	713

8.3.10	Direct SCell Activation of Multiple Downlink SCells at Handover	715

8.3.12	SCell Activation Delay Requirement for Deactivated PUCCH SCell	716

8.3.13	SCell activation delay Requirement for Deactivated PUCCH SCell with Multiple SCells	720

8.3.14	SCell Deactivation Delay Requirement for Activated PUCCH SCell	723

8.3.15	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple Downlink SCells	723

8.3.16	Fast SCell Activation Delay Requirement for Deactivated SCell	723

8.3.17	SCell Activation Delay Requirement for Deactivated SCell with the L3 reporting during activation	726

8.3.18	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells with L3 reporting	729

8.3.19	OD-SSB based SCell Activation Delay Requirement for Deactivated SCell	732

8.3.20	OD-SSB based SCell Deactivation Delay Requirement for Activated SCell	736

8.3.21	OD-SSB based Direct SCell Activation at SCell addition	737

8.3.22	OD-SSB based SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	738

8.3.23	OD-SSB based SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	742

8.3.25	OD-SSB based SCell Deactivation Delay Requirement for Activated PUCCH SCell	743

8.3.26	OD-SSB based SCell Activation Delay Requirement for Deactivated SCell with the L3 reporting during activation	743

8.3A	SCell Activation and Deactivation Delay in Carriers with CCA	744

8.3A.1	Introduction	744

8.3A.2	SCell Activation Delay Requirement for Deactivated SCell	744

8.3A.3	SCell Deactivation Delay Requirement for Activated SCell	748

8.3D	SCell Activation and Deactivation Delay for ATG	749

8.3D.1	Introduction	749

8.3D.2	SCell Activation Delay Requirement for Deactivated SCell	749

8.3D.3	SCell Deactivation Delay Requirement for Activated SCell	754

8.3D.4	Direct SCell Activation at SCell addition	754

8.3D.5	Direct SCell Activation at Handover	755

8.3D.6	Direct SCell Activation at RRC Resume	757

8.3D.7	Fast SCell Activation Delay Requirement for Deactivated SCell	757

8.3D.8	SCell Activation Delay Requirement for Deactivated SCell with the L3 reporting during activation	759

8.4	UE UL carrier RRC reconfiguration delay	761

8.4.1	Introduction	761

8.4.2	UE UL carrier configuration delay requirement	762

8.4.3	UE UL carrier deconfiguration delay requirement	762

8.5	Link Recovery Procedures	762

8.5.1	Introduction	762

8.5.1.1	Introduction of Requirement on Link Recovery Procedures for UE configured with relaxed measurement criteria	763

8.5.2	Requirements for SSB based beam failure detection	764

8.5.2.1	Introduction	764

8.5.2.2	Minimum requirement	765

8.5.2.3	Measurement restriction for SSB based beam failure detection	769

8.5.2.4	Minimum requirement of SSB based beam failure detection for UE fulfilling relaxed measurement criteria	770

8.5.3	Requirements for CSI-RS based beam failure detection	771

8.5.3.1	Introduction	771

8.5.3.2	Minimum requirement	772

8.5.3.3	Measurement restrictions for CSI-RS beam failure detection	776

8.5.3.4	Minimum requirement of CSI-RS based beam failure detection for UE fulfilling relaxed measurement criteria	778

8.5.4	Minimum requirement for L1 indication	778

8.5.5	Requirements for SSB based candidate beam detection	779

8.5.5.1	Introduction	779

8.5.5.2	Minimum requirement	780

8.5.5.3	Measurement restriction for SSB based candidate beam detection	784

8.5.6	Requirements for CSI-RS based candidate beam detection	785

8.5.6.1	Introduction	785

8.5.6.2	Minimum requirement	785

8.5.6.3	Measurement restriction for CSI-RS based candidate beam detection	790

8.5.7	Scheduling availability of UE during beam failure detection	790

8.5.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	791

8.5.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	791

8.5.7.3	Scheduling availability of UE performing beam failure detection on FR2	792

8.5.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	793

8.5.8	Scheduling availability of UE during candidate beam detection	793

8.5.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	793

8.5.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	793

8.5.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	794

8.5.8.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	794

8.5.9	Requirements for Beam Failure Recovery in SCell	795

8.5.9.1	Introduction	795

8.5.9.2	Requirement	795

8.5.10	Minimum requirement at transitions for beam failure detection	795

8.5.11	Minimum requirement under IDC Interference	796

8.5.12	Minimum requirement at transitions for candidate beam detection	796

8.5A	Link Recovery Procedures when CCA is used on target frequency	796

8.5A.1	Introduction	796

8.5A.2	Requirements for SSB based beam failure detection	797

8.5A.2.1	Introduction	797

8.5A.2.2	Minimum requirement	797

8.5A.2.3	Measurement restriction for SSB based beam failure detection	799

8.5A.3	Void	800

8.5A.4	Minimum requirement for L1 indication	800

8.5A.5	Requirements for SSB based candidate beam detection	800

8.5A.5.1	Introduction	800

8.5A.5.2	Minimum requirement	800

8.5A.5.3	Measurement restriction for SSB based candidate beam detection	802

8.5A.6	Void	803

8.5A.7	Scheduling availability of UE during beam failure detection	803

8.5A.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH	803

8.5A.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH	803

8.5A.7.3	Scheduling availability of UE performing beam failure detection on FR2-2	803

8.5A.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR-DC	803

8.5A.8	Scheduling availability of UE during candidate beam detection	803

8.5A.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2-2	804

8.5.8A.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR-DC	804

8.5B	Link Recovery Procedures for Redcap	804

8.5B.1	Introduction	804

8.5B.2	Requirements for SSB based beam failure detection for Redcap	804

8.5B.2.1	Introduction	804

8.5B.2.2	Minimum requirement	805

8.5B.2.3	Measurement restriction for SSB based beam failure detection	806

8.5B.3	Requirements for CSI-RS based beam failure detection for Redcap	807

8.5B.3.1	Introduction	807

8.5B.3.2	Minimum requirement	807

8.5B.3.3	Measurement restrictions for CSI-RS beam failure detection	809

8.5B.4	Minimum requirement for L1 indication for Redcap	810

8.5B.5	Requirements for SSB based candidate beam detection for Redcap	810

8.5B.5.1	Introduction	810

8.5B.5.2	Minimum requirement	810

8.5B.5.3	Measurement restriction for SSB based candidate beam detection	812

8.5B.6	Requirements for CSI-RS based candidate beam detection for Redcap	812

8.5B.6.1	Introduction	812

8.5B.6.2	Minimum requirement	813

8.5B.6.3	Measurement restriction for CSI-RS based candidate beam detection	815

8.5B.7	Scheduling availability of UE during beam failure detection for Redcap	815

8.5B.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	815

8.5B.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	815

8.5B.7.3	Scheduling availability of UE performing beam failure detection on FR2	815

8.5B.8	Scheduling availability of UE during candidate beam detection for Redcap	816

8.5B.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	816

8.5B.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	816

8.5B.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	816

8.5B.9	Minimum requirement at transitions for beam failure detection for Redcap	817

8.5C	Link Recovery Procedures for Satellite Access	817

8.5C.1	Introduction	817

8.5C.2	Requirements for SSB based beam failure detection	817

8.5C.2.1	Introduction	817

8.5C.2.2	Minimum requirement	818

8.5C.2.3	Measurement restriction for SSB based beam failure detection	819

8.5C.3	Requirements for CSI-RS based beam failure detection	819

8.5C.3.1	Introduction	819

8.5C.3.2	Minimum requirement	820

8.5C.3.3	Measurement restrictions for CSI-RS beam failure detection	821

8.5C.4	Minimum requirement for L1 indication	821

8.5C.5	Requirements for SSB based candidate beam detection	822

8.5C.5.1	Introduction	822

8.5C.5.2	Minimum requirement	822

8.5C.5.3	Measurement restriction for SSB based candidate beam detection	823

8.5C.6	Requirements for CSI-RS based candidate beam detection	823

8.5C.6.1	Introduction	823

8.5C.6.2	Minimum requirement	823

8.5C.6.3	Measurement restriction for CSI-RS based candidate beam detection	824

8.5C.7	Scheduling availability of UE during beam failure detection	825

8.5C.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1-NTN	825

8.5C.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1-NTN	825

8.5C.8	Scheduling availability of UE during candidate beam detection	825

8.5C.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1-NTN	825

8.5C.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1-NTN	825

8.5C.9	Minimum requirement at transitions for beam failure detection	825

8.5D	Link Recovery Procedures for ATG	826

8.5D.1	Introduction	826

8.5D.2	Requirements for SSB based beam failure detection	826

8.5D.2.1	Introduction	826

8.5D.2.2	Minimum requirement	826

8.5D.2.3	Measurement restriction for SSB based beam failure detection	828

8.5D.3	Requirements for CSI-RS based beam failure detection	828

8.5D.3.1	Introduction	828

8.5D.3.2	Minimum requirement	828

8.5D.3.3	Measurement restrictions for CSI-RS beam failure detection	830

8.5D.4	Minimum requirement for L1 indication	830

8.5D.5	Requirements for SSB based candidate beam detection	830

8.5D.5.1	Introduction	830

8.5D.5.2	Minimum requirement	830

8.5D.5.3	Measurement restriction for SSB based candidate beam detection	832

8.5D.6	Requirements for CSI-RS based candidate beam detection	832

8.5D.6.1	Introduction	832

8.5D.6.2	Minimum requirement	832

8.5D.6.3	Measurement restriction for CSI-RS based candidate beam detection	834

8.5D.7	Scheduling availability of UE during beam failure detection	834

8.5D.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	834

8.5D.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	834

8.5D.8	Scheduling availability of UE during candidate beam detection	835

8.5D.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	835

8.5D.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	835

8.5D.9	Minimum requirement at transitions for beam failure detection	835

8.5D.10	Requirements for Beam Failure Recovery in SCell	835

8.5E	Link Recovery Procedures for RedCap UE with Satellite Access	835

8.5E.1	Introduction	835

8.5E.2	Requirements for SSB based beam failure detection for RedCap UE with satellite access	836

8.5E.2.1	Introduction	836

8.5E.2.2	Minimum requirement	836

8.5E.2.3	Measurement restrictions for SSB beam failure detection	837

8.5E.3	Requirements for CSI-RS based beam failure detection for RedCap UE with satellite access	837

8.5E.3.1	Introduction	837

8.5E.3.2	Minimum requirement	837

8.5E.3.3	Measurement restrictions for CSI-RS beam failure detection	838

8.5E.4	Minimum requirement for L1 indication for RedCap UE with satellite access	838

8.5E.5	Requirements for SSB based candidate beam detection for RedCap UE with satellite access	838

8.5E.6	Requirements for CSI-RS based candidate beam detection for RedCap UE with satellite access	838

8.5E.7	Scheduling availability of UE during beam failure detection for RedCap UE with satellite access	838

8.5E.8	Scheduling availability of UE during candidate beam detection for RedCap UE with satellite access	838

8.5E.9	Minimum requirement at transitions for beam failure detection for RedCap UE with satellite access	839

8.6	Active BWP switch delay	839

8.6.1	Introduction	839

8.6.2	DCI and timer based BWP switch delay on a single CC	839

8.6.2A	DCI based BWP switch delay on multiple CCs	840

8.6.2A.1	Simultaneous DCI based BWP switch delay on multiple CCs	841

8.6.2A.2	Non-simultaneous DCI based BWP switch delay on multiple CCs	842

8.6.2B	Timer based BWP switch delay on multiple CCs	842

8.6.2B.1	Simultaneous timer based BWP switch delay on multiple CCs	842

8.6.2B.2	Non-simultaneous timer based BWP switch delay on multiple CCs	843

8.6.3	RRC based BWP switch delay on a single CC	844

8.6.3A	RRC based BWP switch delay on multiple CCs	844

8.6.3A.1	Simultaneous RRC based BWP switch delay on multiple CCs	844

8.6.3A.2	Non-simultaneous RRC based BWP switch delay on multiple CCs	845

8.6.4	BWP switch delay on Consistent UL CCA recovery	845

8.6A	Active BWP switch delay for RedCap	846

8.6A.1	Introduction	846

8.6A.2	DCI and timer based BWP switch delay on a single CC	846

8.6A.3	RRC based BWP switch delay on a single CC	847

8.6C	Active BWP switch delay for satellite access	848

8.6C.1	Introduction	848

8.6C.2	DCI and timer based BWP switch delay on a single CC	848

8.6C.3	RRC based BWP switch delay on a single CC	849

8.6D	Active BWP switch delay for ATG	850

8.6D.1	Introduction	850

8.6D.2	DCI and timer based BWP switch delay on a single CC	850

8.6D.2A	DCI based BWP switch delay on multiple CCs	851

8.6D.2A.1	Simultaneous DCI based BWP switch delay on multiple CCs	851

8.6D.2B	Timer based BWP switch delay on multiple CCs	853

8.6D.2B.1	Simultaneous timer based BWP switch delay on multiple CCs	853

8.6D.2B.2	Non-simultaneous timer based BWP switch delay on multiple CCs	853

8.6D.3	RRC based BWP switch delay on a single CC	854

8.6D.3A	RRC based BWP switch delay on multiple CCs	855

8.6D.3A.1	Simultaneous RRC based BWP switch delay on multiple CCs	855

8.6E	Active BWP switch delay for RedCap UE with satellite access	855

8.6E.1	Introduction	855

8.6E.2	DCI and timer based BWP switch delay on a single CC	855

8.6E.3	RRC based BWP switch delay on a single CC	856

8.7	Void	856

8.8	NE-DC: E-UTRAN PSCell Addition and Release Delay	856

8.8.1	Introduction	856

8.8.2	E-UTRAN PSCell Addition Delay Requirement	856

8.8.3	E-UTRAN PSCell Release Delay Requirement	856

8.9	NR-DC: PSCell Addition and Release Delay	857

8.9.1	Introduction	857

8.9.2	PSCell Addition Delay Requirement	857

8.9.3	PSCell Release Delay Requirement	857

8.9A	Conditional PSCell Addition Delay	858

8.9A.1	Introduction	858

8.9A.2	Conditional PSCell Addition Delay Requirement	858

8.9A.2.1	Measurement time	859

8.9B	NR-DC: PSCell Addition and Release Delay in Carriers with CCA	859

8.9B.1	Introduction	859

8.9B.2	PSCell Addition Delay Requirement	859

8.9B.3	PSCell Release Delay Requirement	860

8.9C	Subsequent Conditional PSCell Addition Delay	860

8.9C.1	Introduction	860

8.9C.2	Subsequent Conditional PSCell Addition Delay Requirement	860

8.9C.2.1	Measurement time	861

8.10	Active TCI state switching delay	861

8.10.1	Introduction	861

8.10.2	Known conditions for TCI state	861

8.10.2A	Known conditions for TCI state with beam prediction	862

8.10.3A	MAC-CE based TCI state switch delay in HST FR2 scenarios	863

8.10.4	DCI based TCI state switch delay	864

8.10.5	RRC based TCI state switch delay	864

8.10.6	Active TCI state list update delay	865

8.10A	Active TCI state switching delay with CCA	865

8.10A.1	Introduction	865

8.10A.2	Known conditions for TCI state	865

8.10A.3	MAC-CE based TCI state switch delay	865

8.10A.4	DCI based TCI state switch delay	866

8.10A.5	RRC based TCI state switch delay	867

8.10A.6	Active TCI state list update delay	867

8.10B	Active TCI state switching delay for RedCap	868

8.10B.1	Introduction	868

8.10B.2	Known conditions for TCI state	868

8.10B.3	MAC-CE based TCI state switch delay	868

8.10B.4	DCI based TCI state switch delay	869

8.10B.5	RRC based TCI state switch delay	869

8.10B.6	Active TCI state list update delay	870

8.10C	Active TCI state switching delay for satellite access	870

8.10C.1	Introduction	870

8.10C.2	MAC-CE based TCI state switch delay	870

8.10C.4	DCI based TCI state switch delay	870

8.10C.5	RRC based TCI state switch delay	871

8.10C.6	Active TCI state list update delay	871

8.10D	Active TCI state switching delay for ATG	871

8.10D.2	Void	871

8.10D.6	Active TCI state list update delay	872

8.10E	Active TCI state switching delay for UE operating in FR2-1 and configured with groupBasedBeamReporting-r17	872

8.10E.1	Introduction	872

8.10E.2	Known conditions for TCI state	872

8.10E.3	MAC-CE based dual DL TCI state switch delay	873

8.10E.3.1	MAC-CE based dual DL TCI state switching delay for sDCI	873

8.10E.3.2	MAC-CE based dual DL TCI state switching delay for mDCI	873

8.10E.4	DCI based dual DL TCI state switch delay for sDCI and mDCI	873

8.10E.4.1	DCI based dual DL TCI state switching delay for sDCI	873

8.10E.4.2	DCI based dual DL TCI state switching delay for mDCI	874

8.10E.5	RRC based dual DL TCI state switch delay	874

8.10E.6	Active DL TCI state list update delay	874

8.10E.6.1	Active DL TCI state list update delay for sDCI	874

8.10E.6.2	Active DL TCI state list update delay for mDCI	874

8.10F	Active TCI state switching delay for RedCap UE with satellite access	874

8.10F.1	Introduction	874

8.10F.2	MAC-CE based TCI state switch delay	875

8.10F.4	DCI based TCI state switch delay	875

8.10F.5	RRC based TCI state switch delay	875

8.10F.6	Active TCI state list update delay	875

8.11	PSCell Change	875

8.11A	PSCell Change in Carriers with CCA	875

8.11B	Conditional PSCell Change	876

8.11B.1	Introduction	876

8.11B.2	Conditional PSCell Change delay	876

8.11B.2.1	Measurement time	877

8.11D	Conditional PSCell Change in Carriers with CCA	877

8.11D.1	Introduction	877

8.11D.2	Conditional PSCell Change delay	877

8.11D.2.1	Measurement time	878

8.11E	Subsequent Conditional PSCell Change	878

8.11E.1	Introduction	878

8.11E.2	Subsequent Conditional PSCell Change delay	878

8.11E.2.1	Measurement time	879

8.12	Uplink spatial relation switch delay	879

8.12.1	Introduction	879

8.12.2	Known conditions for spatial relation when associated with DL-RS	880

8.12.3	MAC-CE based spatial relation switch delay	880

8.12.4	DCI based spatial relation switch delay	881

8.12.5	RRC based spatial relation switch delay	881

8.12A	Uplink spatial relation switch delay for RedCap	881

8.12A.1	Introduction	881

8.12A.2	Known conditions for spatial relation when associated with DL-RS	882

8.12A.3	MAC-CE based spatial relation switch delay	882

8.12A.4	DCI based spatial relation switch delay	883

8.12A.5	RRC based spatial relation switch delay	883

8.12C	Uplink spatial relation switch delay for satellite access	883

8.12C.1	Void	884

8.12C.2	Void	884

8.12C.3	Void	884

8.12C.4	Void	884

8.12C.5	Void	884

8.13	UE-specific CBW change	884

8.13.1	Introduction	884

8.13.2	UE-specific CBW change delay	884

8.13A	UE-specific CBW change for RedCap	884

8.13A.1	Introduction	884

8.13A.2	UE-specific CBW change delay	884

8.13C	UE-specific CBW change for satellite access	885

8.13C.1	Introduction	885

8.13C.2	UE-specific CBW change delay	885

8.13D	UE-specific CBW change for ATG	885

8.13D.1	Introduction	885

8.13D.2	UE-specific CBW change delay	885

8.13E	UE-specific CBW change for RedCap UE with satellite access	886

8.13E.1	Introduction	886

8.13E.2	UE-specific CBW change delay	886

8.14	Pathloss reference signal switching delay	886

8.14.1	Introduction	886

8.14.2	Known conditions for pathloss reference signal	886

8.14.3	MAC-CE based pathloss reference signal switch delay	886

8.14C	Pathloss reference signal switching delay for satellite access	887

8.14C.1	Introduction	887

8.14C.2	Known conditions for pathloss reference signal	888

8.14C.3	MAC-CE based pathloss reference signal switch delay	888

8.14D	Pathloss reference signal switching delay for ATG	888

8.14D.1	Introduction	888

8.14D.2	Known conditions for pathloss reference signal	888

8.14D.3	MAC-CE based pathloss reference signal switch delay	889

8.14E	Pathloss reference signal switching delay for RedCap UE with satellite access	889

8.14E.1	Introduction	889

8.14E.2	Known conditions for pathloss reference signal	889

8.14E.3	MAC-CE based pathloss reference signal switch delay	889

8.15	Active downlink TCI state switching delay for unified TCI	889

8.15.1	Introduction	889

8.15.4	DCI based downlink TCI state switch delay	891

8.15.5	Active Downlink TCI state list update delay	891

8.15D	Active downlink TCI state switching delay for unified TCI for ATG	892

8.15D.1	Introduction	892

8.15D.2	Void	892

8.15D.4	DCI based downlink TCI state switch delay	893

8.15D.5	Active Downlink TCI state list update delay	893

8.16	Active uplink TCI state switching delay for unified TCI	893

8.16.1	Introduction	893

8.16.3	MAC-CE based uplink TCI state switch delay	894

8.16.4	DCI based uplink TCI state switch delay	896

8.16.5	Active Uplink TCI state list update delay	897

8.16D	Active uplink TCI state switching delay for unified TCI for ATG	898

8.16D.1	Introduction	898

8.16D.2	Void	898

8.16D.3	MAC-CE based uplink TCI state switch delay	898

8.16D.4	DCI based uplink TCI state switch delay	899

8.16D.5	Active Uplink TCI state list update delay	899

8.17	SCG Activation and Deactivation Delay	900

8.17.1	Introduction	900

8.17.2	SCG Activation Delay Requirement	900

8.17.3	SCG Deactivation Delay Requirement	901

8.18	TRP specific Link Recovery Procedures	901

8.18.1	Introduction	901

8.18.2	Requirements for TRP specific SSB based beam failure detection	902

8.18.2.1	Introduction	902

8.18.2.2	Minimum requirement	903

8.18.2.3	Measurement restriction for SSB based beam failure detection	905

8.18.3	Requirements for CSI-RS based beam failure detection	906

8.18.3.1	Introduction	906

8.18.3.2	Minimum requirement	906

8.18.3.3	Measurement restrictions for CSI-RS beam failure detection	909

8.18.4	Minimum requirement for L1 indication	911

8.18.5	Requirements for SSB based candidate beam detection	911

8.18.5.1	Introduction	911

8.18.5.2	Minimum requirement	911

8.18.5.3	Measurement restriction for SSB based candidate beam detection	913

8.18.6	Requirements for CSI-RS based candidate beam detection	914

8.18.6.1	Introduction	914

8.18.6.2	Minimum requirement	914

8.18.6.3	Measurement restriction for CSI-RS based candidate beam detection	917

8.18.7	Requirements for TRP specific Beam Failure Recovery	918

8.18.7.1	Introduction	918

8.18.7.2	Requirement	918

8.18.8	Scheduling availability of UE during TRP specific beam failure detection	918

8.18.8.1	Scheduling availability of UE performing TRP specific beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	918

8.18.8.2	Scheduling availability of UE performing TRP specific beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	919

8.18.8.3	Scheduling availability of UE performing TRP specific beam failure detection on FR2	919

8.18.8.4	Scheduling availability of UE performing TRP specific beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	920

8.18.9	Scheduling availability of UE during TRP specific candidate beam detection	920

8.18.9.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	920

8.18.9.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	920

8.18.9.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	920

8.18.9.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	921

8.19	Pre-configured measurement gap activation/deactivation delay	921

8.19.1	Introduction	921

8.19.2	Pre-configured measurement gap activation/deactivation upon DCI/timer-based BWP switch	921

8.19.2.1	Activation/deactivation upon DCI/timer-based BWP switch delay on a single CC	921

8.19.3	Pre-configured measurement gap activation/deactivation upon SCell activation/deactivation	921

8.19.4	Pre-configured measurement gap activation/deactivation upon RRC reconfiguration	922

8.19.5	Activation/deactivation delay requirements for concurrent measurement gaps with Pre-MG	922

8.19.5.1	Activation/deactivation delay requirements for non-overlapped activation/deactivation of concurrent measurement gaps with Pre- MG	922

8.19.5.2	Activation/deactivation delay requirements for fully overlapped activation/deactivation of concurrent measurement gaps with Pre- MG	922

8.19.5.3	Pre-MG activation/deactivation delay when colliding with a concurrent measurement gap	922

8.19D	Pre-configured measurement gap activation/deactivation delay for ATG	923

8.19D.1	Introduction	923

8.19D.2	Pre-configured measurement gap activation/deactivation upon DCI/timer-based BWP switch	923

8.19D.2.1	Activation/deactivation upon DCI/timer-based BWP switch delay on a single CC	923

8.19D.3	Pre-configured measurement gap activation/deactivation upon RRC reconfiguration	923

8.19D.4	Pre-configured measurement gap activation/deactivation upon SCell activation/deactivation	923

8.20	LTM PSCell Cell Switch	924

8.20.1	Introduction	924

8.20.2	LTM Cell Switch delay	925

8.20.3	Void	925

8.21	Active downlink TCI state switching delay for unified TCI for single-DCI mTRP	925

8.21.1	Introduction	925

8.21.2	Known conditions for downlink TCI state	925

8.21.3	MAC-CE based downlink TCI state switch delay	925

8.21.4	DCI based downlink TCI state switch delay	927

8.21.5	Active Downlink TCI state list update delay	927

8.22	Active downlink TCI state switching delay for unified TCI for multi-DCI mTRP	928

8.22.1	Introduction	928

8.22.2	Known conditions for downlink TCI state	928

8.22.3	MAC-CE based downlink TCI state switch delay	928

8.22.4	DCI based downlink TCI state switch delay	929

8.22.5	Active Downlink TCI state list update delay	930

8.23	Active uplink TCI state switching delay for unified TCI for single-DCI mTRP	930

8.23.1	Introduction	930

8.23.2	Known conditions for uplink TCI state	931

8.23.3	MAC-CE based uplink TCI state switch delay	931

8.23.4	DCI based uplink TCI state switch delay	933

8.23.5	Active uplink TCI state list update delay	933

8.24	Active uplink TCI state switching delay for unified TCI for multi-DCI mTRP	934

8.24.1	Introduction	934

8.24.2	Known conditions for uplink TCI state	934

8.24.3	MAC-CE based uplink TCI state switch delay	934

8.24.4	DCI based uplink TCI state switch delay	936

8.24.5	Active Uplink TCI state list update delay	936

8.25	TCI state activation for LTM candidate cell	937

8.25.1	Introduction	937

8.25.2	Known TCI state conditions	937

8.25.3	SSB based TCI state activation delay	938

9	Measurement Procedure	939

9.1	General measurement requirement	939

9.1.1	Introduction	939

9.1.2	Measurement gap	939

9.1.2.1	EN-DC: Measurement Gap Sharing	949

9.1.2.1a	SA: Measurement Gap Sharing	949

9.1.2.1b	NE-DC: Measurement Gap Sharing	950

9.1.2.1c	NR-DC: Measurement Gap Sharing	951

9.1.3	UE Measurement capability	952

9.1.3.1	EN-DC: Monitoring of multiple layers using gaps	952

9.1.3.1a	SA: Monitoring of multiple layers using gaps	953

9.1.3.1b	NE-DC: Monitoring of multiple layers using gaps	953

9.1.3.1c	NR-DC: Monitoring of multiple layers using gaps	954

9.1.3.2	EN-DC: Maximum allowed layers for multiple monitoring	954

9.1.3.2a	SA: Maximum allowed layers for multiple monitoring	955

9.1.3.2b	NE-DC: Maximum allowed layers for multiple monitoring	956

9.1.3.2c	NR-DC: Maximum allowed layers for multiple monitoring	956

9.1A.3.2	Void	957

9.1.3A	UE Measurement capability under operation mode with CCA	957

9.1.3A.1	EN-DC: Monitoring of multiple layers using gaps under CCA	957

9.1.3A.1a	SA: Monitoring of multiple layers using gaps under CCA	957

9.1.3A.2	EN-DC: Maximum allowed layers for multiple monitoring under CCA	957

9.1.3A.2a	SA: Maximum allowed layers for multiple monitoring under CCA	958

9.1.3C	UE Measurement capability under operation mode with satellite access	958

9.1.3C.1a	SA: Monitoring of multiple layers using gaps under satellite access	958

9.1.3C.2a	SA: Maximum allowed layers for multiple monitoring for SAN	959

9.1.4	Capabilities for Support of Event Triggering and Reporting Criteria	959

9.1.4.1	Introduction	959

9.1.4.2	Requirements	959

9.1.5	Carrier-specific scaling factor	962

9.1.5.1	Monitoring of multiple layers outside gaps	962

9.1.5.1.1	EN-DC mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	965

9.1.5.1.2	SA mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	970

9.1.5.1.3	NR-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed outside gaps	972

9.1.5.1.4	NE-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based measurements performed outside gaps	974

9.1.5.2	Monitoring of multiple layers within gaps	976

9.1.5.2.1	EN-DC mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	978

9.1.5.2.2	SA mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	980

9.1.5.2.3	NE-DC: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed within gaps	982

9.1.5.2.4	NR-DC: carrier-specific scaling factor for SSB-based and CSI-RS-based L3 measurements performed within gaps	984

9.1.5.2.5	SA mode: carrier-specific scaling factor for PRS-based measurements performed within gaps	986

9.1.5.2.6	NE-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	986

9.1.5.2.7	NR-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	986

9.1.5.3	Monitoring of multiple layers within NCSG	987

9.1.5.3.1	SA mode: carrier-specific scaling factor for measurements performed within NCSG	988

9.1.5.4	L1-RSRP measurements within measurement gap	989

9.1.5.4.1	SA mode: carrier-specific scaling factor for L1-RSRP measurements performed within measurement gap	990

9.1.5.4.2	NR-DC: carrier-specific scaling factor for L1-RSRP measurements performed within measurement gap	991

9.1.6	Minimum requirement at transitions	993

9.1.7	Pre-configured measurement gap	993

9.1.7.1	Introduction	993

9.1.7.2	Requirements applicability	994

9.1.7.3	Requirements	994

9.1.7.3.1	Requirements for autonomous activation/deactivation mechanism	994

9.1.7.3.2	Requirements for network-controlled activation/deactivation mechanism	995

9.1.7.3.3	Requirements for reception/transmission during activation/deactivation	996

9.1.8	Concurrent measurement gaps	996

9.1.8.1	Introduction	996

9.1.8.2	Requirements	996

9.1.8.3	Collision between concurrent measurement gaps	997

9.1.8.4	Measurement gap related requirements of concurrent measurement gaps	997

9.1.9	Network controlled small gap	998

9.1.9.1	Introduction	998

9.1.9.2	Requirements applicability	999

9.1.10	MUSIM gaps	1001

9.1.10.1	Introduction	1002

9.1.10.2	Priorities for MUSIM gaps	1003

9.1.10.3	Keep solution for MUSIM gaps	1003

9.1.10.4	Collisions between different MUSIM gaps	1003

9.1.10.5	Collisions between MUSIM gaps and measurement gaps	1003

9.1.10.6	MUSIM gap related requirements	1004

9.1.11	UL gap for Tx power management	1004

9.1.12	Concurrent measurement gaps with Pre-MG	1004

9.1.12.1	Introduction	1004

9.1.12.2	Requirements	1005

9.1.12.3	Collisions involving Pre-MG(s)	1005

9.1.12.4	Collision between Pre-MG activation/deactivation and measurement gap	1006

9.1.12.5	Pre-MG related requirements	1006

9.1.13	Concurrent measurement gaps with NCSG	1006

9.1.13.1	Introduction	1006

9.1.13.2	Requirements	1007

9.1.13.3	Collision involving NCSGs	1008

9.1.14	Measurement gap occasion cancellation	1008

9.1.14.1	Introduction	1008

9.1.14.2	Applicable measurement gap configurations	1008

9.1.14.3	Applicability	1009

9.1.14.4	Requirements for cancelling measurement gap occasions	1009

9.1A	General measurement requirement for RedCap	1009

9.1A.1	Introduction	1009

9.1A.2	Measurement gap	1009

9.1A.2.1	SA: Measurement Gap Sharing	1013

9.1A.3	UE Measurement capability	1014

9.1A.3.1	SA: Monitoring of multiple layers using gaps	1014

9.1A.3.2	SA: Maximum allowed layers for multiple monitoring	1014

9.1A.4	Capabilities for Support of Event Triggering and Reporting Criteria	1014

9.1A.4.1	Introduction	1014

9.1A.4.2	Requirements	1015

9.1A.5	Carrier-specific scaling factor	1015

9.1A.5.1	Monitoring of multiple layers outside gaps	1015

9.1A.5.1.1	SA mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps	1016

9.1A.5.2	Monitoring of multiple layers within gaps	1016

9.1A.5.2.1	SA mode: carrier-specific scaling factor for SSB measurements performed within gaps	1016

9.1A.6	Minimum requirement at transitions	1018

9.1C	General measurement requirement for SAN	1018

9.1C.1	Introduction	1018

9.1C.2	Measurement gap	1019

9.1C.8	Concurrent measurement gaps for SAN	1021

9.1C.8.1	Introduction	1021

9.1C.8.2	Requirements	1021

9.1C.8.3	Collision between concurrent measurement gaps	1022

9.1C.8.4	Measurement gap related requirements of concurrent measurement gaps	1022

9.1C.9	Collision between SMTC and measurement gap for SAN	1022

9.1C.9.1	Introduction	1022

9.1C.9.2	Collision between SMTCs and measurement gap	1022

9.1C.9.3	Collision between multiple SMTCs on a SAN carrier	1023

9.1D	General measurement requirement for ATG	1023

9.1D.1	Introduction	1023

9.1D.2	Measurement gap	1023

9.1D.2.1a	SA: Measurement Gap Sharing	1026

9.1D.3	UE Measurement capability	1026

9.1D.3.1	SA: Monitoring of multiple layers using gaps	1026

9.1D.3.2	SA: Maximum allowed layers for multiple monitoring	1026

9.1D.4	Void	1027

9.1D.5	Carrier-specific scaling factor	1027

9.1D.5.1	Monitoring of multiple layers outside gaps	1027

9.1D.5.1.1	Void	1027

9.1D.5.1.2	SA mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements performed outside gaps	1027

9.1D.5.2	Monitoring of multiple layers within gaps	1028

9.1D.5.2.1	Void	1029

9.1D.5.2.2	SA mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements performed within gaps	1029

9.1D.6	Void	1029

9.1D.7	Pre-configured measurement gap	1029

9.1D.7.1	Introduction	1029

9.1D.7.2	Requirements applicability	1030

9.1D.7.3	Requirements	1030

9.1D.7.3.1	Requirements for autonomous activation/deactivation mechanism	1030

9.1D.7.3.2	Requirements for network-controlled activation/deactivation mechanism	1031

9.1D.7.3.3	Requirements for reception/transmission during activation/deactivation	1031

9.1D.8	Capabilities for Support of Event Triggering and Reporting Criteria	1031

9.1D.8.1	Introduction	1031

9.1D.8.2	Requirements	1032

9.1D.9	Minimum requirement at transitions	1032

9.1E	General measurement requirement for RedCap with satellite access	1032

9.1E.1	Introduction	1032

9.1E.2	Measurement gap	1033

9.1E.8	Concurrent measurement gaps for RedCap with SAN	1033

9.1E.8.1	Introduction	1033

9.1E.8.2	Requirements	1033

9.1E.8.3	Collision between concurrent measurement gaps	1033

9.1E.8.4	Measurement gap related requirements of concurrent measurement gaps	1033

9.1E.9	Collision between SMTC and measurement gap for RedCap with satellite access	1033

9.1E.9.1	Introduction	1033

9.1E.9.2	Collision between SMTCs and measurement gap	1033

9.1E.9.3	Collision between multiple SMTCs on a SAN carrier	1034

9.2	NR intra-frequency measurements	1034

9.2.1	Introduction	1034

9.2.2	Requirements applicability	1037

9.2.3	Number of cells and number of SSB	1038

9.2.3.1	Requirements for FR1	1038

9.2.3.2	Requirements for FR2	1038

9.2.4	Measurement Reporting Requirements	1039

9.2.4.1	Periodic Reporting	1039

9.2.4.2	Event-triggered Periodic Reporting	1039

9.2.4.3	Event Triggered Reporting	1039

9.2.4.4	SCell activation Triggered Reporting	1040

9.2.5	Intrafrequency measurements without measurement gaps	1040

9.2.5.1	Intra-frequency cell identification	1040

9.2.5.2	Measurement period	1048

9.2.5.3	Scheduling availability of UE during intra-frequency measurements	1052

9.2.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	1052

9.2.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	1053

9.2.5.3.3	Scheduling availability of UE performing measurements on FR2	1054

9.2.5.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	1056

9.2.5.4	SFTD Measurements between PCell and PSCell	1056

9.2.5.4.1	Introduction	1056

9.2.5.4.2	SFTD Measurement delay	1056

9.2.5.4.3	SFTD Measurement Reporting Delay	1057

9.2.6	Intra-frequency measurements with measurement gaps	1057

9.2.6.1	Void	1057

9.2.6.2	Intra-frequency cell identification	1057

9.2.6.3	Intra-frequency Measurement Period	1061

9.2.7	Intra-frequency measurements with NCSG	1063

9.2.7.1	Intra-frequency cell identification	1063

9.2.7.2	Measurement period	1065

9.2.7.3	Scheduling availability during intra-frequency measurement with NCSG	1066

9.2A	NR intra-frequency measurements with CCA	1067

9.2A.1	Introduction	1067

9.2A.2	Requirements applicability	1067

9.2A.3	Number of cells and number of SSB	1068

9.2A.3.1	Requirements for FR1	1068

9.2A.3.2	Requirements for FR2-2	1068

9.2A.4	Measurement Reporting Requirements	1068

9.2A.5	Intra-frequency measurements without measurement gaps	1069

9.2A.5.2	Measurement period	1073

9.2A.5.3	Scheduling availability of UE during intra-frequency measurements	1075

9.2A.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	1076

9.2A.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	1076

9.2A.5.3.3	Scheduling availability of UE performing measurements in TDD bands on FR2-2	1076

9.2A.6	Intra-frequency measurements with measurement gaps	1077

9.2A.6.1	Intra-frequency cell identification	1077

9.2A.6.2	Intra-frequency Measurement Period	1079

9.2A.7	Intra-frequency RSSI and Channel occupancy measurements	1080

9.2A.7.1	Intra-frequency RSSI measurements	1080

9.2A.7.2	Intra-frequency Channel occupancy measurements	1082

9.2A.7.3	Scheduling restriction during RSSI and Channel Occupancy measurements in FR1	1084

9.2A.7.4	Scheduling restriction during RSSI measurements in FR2-2	1084

9.2B	NR intra-frequency measurements for RedCap	1084

9.2B.1	Introduction	1084

9.2B.2	Requirements applicability	1085

9.2B.3	Number of cells and number of SSB	1085

9.2B.3.1	Requirements for FR1	1085

9.2B.3.2	Requirements for FR2	1085

9.2B.4	Measurement Reporting Requirements	1086

9.2B.4.1	Periodic Reporting	1086

9.2B.4.2	Event-triggered Periodic Reporting	1086

9.2B.4.3	Event Triggered Reporting	1086

9.2B.5	Intra-frequency measurements without measurement gaps	1087

9.2B.5.1	Intra-frequency cell identification	1087

9.2B.5.2	Measurement period	1089

9.2B.5.3	Scheduling availability of UE during intra-frequency measurements	1090

9.2B.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	1090

9.2B.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	1090

9.2B.5.3.3	Scheduling availability of UE performing measurements on FR2	1091

9.2B.5.3.4	Scheduling availability of HD-FDD UE performing measurements on FR1	1091

9.2B.6	Intra-frequency measurements with measurement gaps	1092

9.2B.6.1	Intra-frequency cell identification	1092

9.2B.6.2	Intra-frequency Measurement Period	1093

9.2C	NR intra-frequency measurements for SAN	1094

9.2C.1	Introduction	1094

9.2C.2	Requirements applicability	1095

9.2C.3	Number of cells and number of SSB	1095

9.2C.3.1	Requirements for FR1-NTN	1095

9.2C.4	Measurement Reporting Requirements	1096

9.2C.4.1	Periodic Reporting	1096

9.2C.4.2	Event-triggered Periodic Reporting	1096

9.2C.4.3	Event Triggered Reporting	1096

9.2C.5	Intra-frequency measurements without measurement gaps	1096

9.2C.5.1	Intra-frequency cell identification	1096

9.2C.5.2	Measurement period	1099

9.2C.5.3	Scheduling availability of UE during intra-frequency measurements	1099

9.2C.5.3.1	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1-NTN	1099

9.2C.5.3.2	Scheduling availability of UE performing measurements on a neighbor cell served by a different satellite in LEO	1099

9.2C.6	Intra-frequency measurements with measurement gaps	1100

9.2C.6.1	Void	1100

9.2C.6.2	Intra-frequency cell identification	1100

9.2C.6.3	Intrafrequency Measurement Period	1102

9.2C.7	Intra-frequency measurements without measurement gaps for NTN band above 10 GHz	1102

9.2C.7.1	Intra-frequency cell identification	1102

9.2C.7.2	Measurement period	1103

9.2C.7.3	Scheduling availability of UE during intra-frequency measurements	1104

9.2C.7.3.1	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on NTN bands above 10 GHz	1104

9.2C.8	Intra-frequency measurements with measurement gaps for NTN band above 10 GHz	1104

9.2C.8.1	Intra-frequency cell identification	1104

9.2C.8.3	Intra-frequency Measurement Period	1105

9.2D	NR intra-frequency measurements for ATG	1106

9.2D.1	Introduction	1106

9.2D.2	Requirements applicability	1106

9.2D.3	Number of cells and number of SSB	1106

9.2D.3.1	Requirements for FR1	1106

9.2D.4	Measurement Reporting Requirements	1107

9.2D.4.1	Periodic Reporting	1107

9.2D.4.2	Event-triggered Periodic Reporting	1107

9.2D.4.3	Event Triggered Reporting	1107

9.2D.4.4	SCell activation Triggered Reporting	1107

9.2D.5	Intra-frequency measurements without measurement gaps	1108

9.2D.5.1	Intra-frequency cell identification	1108

9.2D.5.2	Measurement period	1111

9.2D.5.3	Scheduling availability of UE during intra-frequency measurements	1112

9.2D.5.3.1	Scheduling availability of UE performing measurements on FR1	1112

9.2D.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	1114

9.2D.6	Intra-frequency measurements with measurement gaps	1114

9.2D.6.1	Void	1114

9.2D.6.2	Intra-frequency cell identification	1114

9.2D.6.3	Intra-frequency Measurement Period	1115

9.2E	NR intra-frequency measurements for RedCap with SAN	1116

9.2E.1	Introduction	1116

9.2E.2	Requirements applicability	1116

9.2E.3	Number of cells and number of SSB	1117

9.2E.3.1	Requirements for FR1	1117

9.2E.4	Measurement Reporting Requirements	1117

9.2E.4.1	Periodic Reporting	1117

9.2E.4.2	Event-triggered Periodic Reporting	1117

9.2E.4.3	Event Triggered Reporting	1117

9.2E.5	Intra-frequency measurements without measurement gaps	1117

9.2E.5.1	Intra-frequency cell identification	1117

9.2E.5.2	Measurement period	1118

9.2E.5.3	Scheduling availability of UE during intra-frequency measurements	1118

9.2E.5.3.1	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	1118

9.2E.5.3.2	Scheduling availability of UE performing measurements on a neighbor cell served by a different satellite in LEO	1118

9.2E.5.3.4	Scheduling availability of UE performing measurements in HD-FDD bands on FR1	1118

9.2E.6	Intra-frequency measurements with measurement gaps	1119

9.2E.6.1	Intra-frequency cell identification	1119

9.2E.6.2	Intra-frequency Measurement Period	1119

9.3	NR inter-frequency measurements	1119

9.3.1	Introduction	1119

9.3.2	Requirements applicability	1122

9.3.2.1	Void	1123

9.3.2.2	Void	1123

9.3.3	Number of cells and number of SSB	1123

9.3.3.1	Requirements for FR1	1123

9.3.3.2	Requirements for FR2	1123

9.3.4	Inter-frequency measurement with measurement gaps	1123

9.3.4.1	Void	1129

9.3.4.2	Void	1129

9.3.5	Inter-frequency measurements	1129

9.3.5.1	Void	1132

9.3.5.2	Void	1132

9.3.5.3	Void	1132

9.3.6	Inter-frequency measurements reporting requirements	1132

9.3.6.1	Periodic Reporting	1132

9.3.6.2	Event-triggered Periodic Reporting	1132

9.3.6.3	Event-triggered Reporting	1132

9.3.7	Void	1133

9.3.8	Inter-frequency SFTD measurement requirements	1133

9.3.8.1	Introduction	1133

9.3.8.2	SFTD Measurement delay	1133

9.3.8.3	SFTD Measurement reporting delay	1134

9.3.9	Inter-frequency measurements without measurement gaps	1134

9.3.9.1	Inter-frequency Cell identification	1134

9.3.9.2	Measurement period	1139

9.3.9.3	Scheduling availability of UE during inter-frequency measurements when the SSB is completely contained in the active BWP of the UE	1141

9.3.9.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	1142

9.3.9.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	1143

9.3.9.3.3	Scheduling availability of UE performing measurements on FR2	1144

9.3.9.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	1144

9.3.9.4	Scheduling availability of UE during inter-frequency measurements when the SSB is not completely contained in the active BWP of the UE	1144

9.3.9.4.1	Scheduling availability of UE performing measurements in TDD bands on FR1	1145

9.3.9.4.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	1145

9.3.9.4.3	Scheduling availability of UE performing measurements on FR2	1146

9.3.9.4.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	1147

9.3.10	Inter-frequency measurement with NCSG	1148

9.3.10.1	Inter-frequency cell identification	1148

9.3.10.2	Measurement period	1150

9.3.10.3	Scheduling availability during inter-frequency measurement with NCSG	1150

9.3.10.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	1150

9.3.10.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	1151

9.3.10.3.3	Scheduling availability of UE performing measurements on FR2	1151

9.3.10.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	1153

9.3A	NR inter-frequency measurements in carrier frequencies with CCA	1153

9.3A.1	Introduction	1153

9.3A.2	Requirements applicability	1154

9.3A.3	Number of cells and number of SSB	1154

9.3A.3.1	Requirements for FR1	1154

9.3A.3.2	Requirements for FR2-2	1154

9.3A.4	Inter-frequency cell identification	1155

9.3A.5	Inter-frequency measurements	1157

9.3A.6	Inter-frequency measurements reporting requirements	1158

9.3A.6.1	Periodic Reporting	1158

9.3A.6.2	Event-triggered Periodic Reporting	1158

9.3A.6.3	Event-triggered Reporting	1158

9.3A.8	Inter-frequency RSSI measurements	1159

9.3A.9	Inter-frequency channel occupancy measurements	1160

9.3B	NR inter-frequency measurements for RedCap	1160

9.3B.1	Introduction	1160

9.3B.2	Requirements applicability	1161

9.3B.3	Number of cells and number of SSB	1161

9.3B.3.1	Requirements for FR1	1161

9.3B.3.2	Requirements for FR2	1161

9.3B.4	Inter-frequency measurement with measurement gaps	1161

9.3B.5	Inter-frequency measurements	1163

9.3B.6	Inter-frequency measurements reporting requirements	1164

9.3B.6.1	Periodic Reporting	1164

9.3B.6.2	Event-triggered Periodic Reporting	1164

9.3B.6.3	Event-triggered Reporting	1164

9.3B.7	Inter-frequency measurements without measurement gaps	1164

9.3B.7.1	Inter-frequency Cell identification	1164

9.3B.7.2	Measurement period	1166

9.3B.7.3	Scheduling availability of UE during inter-frequency measurements	1167

9.3B.7.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	1167

9.3B.7.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	1168

9.3B.7.3.3	Scheduling availability of UE performing measurements on FR2	1168

9.3B.7.3.4	Scheduling availability of HD-FDD UE performing measurements on FR1	1168

9.3C	NR inter-frequency measurements for SAN	1169

9.3C.1	Introduction	1169

9.3C.2	Requirements applicability	1170

9.3C.3	Number of cells and number of SSB	1170

9.3C.3.1	Requirements for FR1-NTN	1170

9.3C.4	Inter-frequency measurement with measurement gaps	1170

9.3C.5	Inter-frequency measurements	1172

9.3C.6	Inter-frequency measurements reporting requirements	1172

9.3C.6.1	Periodic Reporting	1172

9.3C.6.2	Event-triggered Periodic Reporting	1173

9.3C.6.3	Event-triggered Reporting	1173

9.3C.7	Inter-frequency measurements without measurement gaps	1173

9.3C.7.1	Inter-frequency Cell identification	1173

9.3C.7.2	Measurement period	1175

9.3C.7.3	Scheduling availability of UE during inter-frequency measurements	1175

9.3C.7.3.1	Void	1176

9.3C.7.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1-NTN	1176

9.3C.7.3.3	Scheduling availability of UE performing measurements on a neighbor cell served by a different satellite in LEO	1176

9.3C.8	Inter-frequency measurement with measurement gaps for NTN band above 10 GHz	1176

9.3C.9	Inter-frequency measurements for NTN band above 10 GHz	1177

9.3C.10	Inter-frequency measurements without measurement gaps for NTN band above 10 GHz	1177

9.3C.10.1	Inter-frequency Cell identification	1177

9.3C.10.2	Measurement period	1178

9.3C.10.3	Scheduling availability of UE during inter-frequency measurements	1179

9.3C.10.3.1	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on NTN bands above 10 GHz	1179

9.3D	NR inter-frequency measurements for ATG	1179

9.3D.1	Introduction	1179

9.3D.2	Requirements applicability	1180

9.3D.3	Number of cells and number of SSB	1180

9.3D.3.1	Requirements for FR1	1180

9.3D.4	Inter-frequency measurement with measurement gaps	1180

9.3D.5	Inter-frequency measurements	1181

9.3D.6	Inter-frequency measurements reporting requirements	1182

9.3D.6.1	Periodic Reporting	1182

9.3D.6.2	Event-triggered Periodic Reporting	1182

9.3D.6.3	Event-triggered Reporting	1182

9.3D.7	Void	1183

9.3D.8	Void	1183

9.3D.9	Inter-frequency measurements without measurement gaps	1183

9.3D.9.1	Inter-frequency Cell identification	1183

9.3D.9.2	Measurement period	1185

9.3D.9.3	Scheduling availability of UE during inter-frequency measurements	1185

9.3D.9.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	1185

9.3D.9.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	1186

9.3E	NR inter-frequency measurements for Redcap UEs with satellite access	1187

9.3E.1	Introduction	1187

9.3E.2	Requirements applicability	1187

9.3E.3	Number of cells and number of SSB	1188

9.3E.3.1	Requirements for FR1	1188

9.3E.4	Inter-frequency measurement with measurement gaps	1188

9.3E.5	Inter-frequency measurements	1189

9.3E.6	Inter-frequency measurements reporting requirements	1189

9.3E.6.1	Periodic Reporting	1189

9.3E.6.2	Event-triggered Periodic Reporting	1189

9.3E.6.3	Event-triggered Reporting	1189

9.3E.7	Inter-frequency measurements without measurement gaps	1190

9.3E.7.1	Inter-frequency Cell identification	1190

9.3E.7.2	Measurement period	1191

9.3E.7.3	Scheduling availability of UE during inter-frequency measurements	1191

9.3E.7.3.1	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	1191

9.3E.7.3.2	Scheduling availability of UE performing measurements in HD-FDD bands on FR1	1191

9.4	Inter-RAT measurements	1191

9.4.1	Introduction	1191

9.4.2	NR − E-UTRAN FDD measurements	1194

9.4.2.1	Introduction	1194

9.4.2.2	Requirements when no DRX is used	1194

9.4.2.3	Requirements when DRX is used	1196

9.4.2.4	Measurement reporting requirements	1198

9.4.2.4.1	Periodic Reporting	1198

9.4.2.4.2	Event-Triggered Periodic Reporting	1198

9.4.2.4.3	Event-Triggered Reporting	1199

9.4.2.5	Scheduling Availability During NR − E-UTRAN FDD measurements with NCSG	1199

9.4.3	NR − E-UTRAN TDD measurements	1199

9.4.3.1	Introduction	1199

9.4.3.2	Requirements when no DRX is used	1199

9.4.3.3	Requirements when DRX is used	1202

9.4.3.4	Measurement reporting requirements	1204

9.4.3.4.1	Periodic Reporting	1204

9.4.3.4.2	Event-Triggered Periodic Reporting	1204

9.4.3.4.3	Event-Triggered Reporting	1205

9.4.3.5	Scheduling Availability During NR − E-UTRAN TDD measurements with NCSG	1205

9.4.4	Inter-RAT RSTD measurements	1205

9.4.4.1	NR − E-UTRAN FDD RSTD measurements	1205

9.4.4.1.1	Introduction	1205

9.4.4.1.2	Requirements	1206

9.4.4.2	NR − E-UTRAN TDD RSTD measurements	1208

9.4.4.2.1	Introduction	1208

9.4.4.2.2	Requirements	1209

9.4.5	Inter-RAT E-CID measurements	1212

9.4.5.1	NR−E-UTRAN FDD E-CID RSRP and RSRQ measurements	1212

9.4.5.1.1	Introduction	1212

9.4.5.1.2	Requirements	1212

9.4.5.1.3	Measurement Reporting Delay	1212

9.4.5.2	NR−E-UTRAN TDD E-CID RSRP and RSRQ measurements	1213

9.4.5.2.1	Introduction	1213

9.4.5.2.2	Requirements	1213

9.4.5.2.3	Measurement Reporting Delay	1213

9.4.6	NR − UTRAN FDD measurements	1213

9.4.6.1	Introduction	1213

9.4.6.2	Requirements when no DRX is used	1213

9.4.6.3	Requirements when DRX is used	1214

9.4.7	NR – E-UTRAN measurements with autonomous gaps	1216

9.4.7.1	CGI identification of an E-UTRA cell with autonomous gaps	1216

9.4.7.2	CGI reporting delay	1216

9.4.8	NR – E-UTRAN measurements without measurement gaps	1217

9.4.8.1	Introduction	1217

9.4.8.2	General requirements	1217

9.4.8.3	NR − E-UTRAN FDD measurements	1218

9.4.8.3.1	Introduction	1218

9.4.8.3.2	Requirements when no DRX is used	1218

9.4.8.3.3	Requirements when DRX is used	1219

9.4.8.3.4	Measurement reporting requirements	1219

9.4.8.3.5	Scheduling availability during NR − E-UTRAN FDD measurements	1220

9.4.8.4	NR − E-UTRAN TDD measurements	1220

9.4.8.4.1	Introduction	1220

9.4.8.4.2	Requirements when no DRX is used	1221

9.4.8.4.3	Requirements when DRX is used	1222

9.4.8.4.4	Measurement reporting requirements	1223

9.4.8.4.5	Scheduling availability during NR − E-UTRAN TDD measurements	1223

9.4A	Inter-RAT measurements for RedCap	1224

9.4A.1	Introduction	1224

9.4A.2	NR − E-UTRAN FDD measurements	1225

9.4A.2.1	Introduction	1225

9.4A.2.2	Requirements when no DRX is used	1225

9.4A.2.3	Requirements when DRX is used	1226

9.4A.2.4	Measurement reporting requirements	1227

9.4A.2.4.1	Periodic Reporting	1227

9.4A.2.4.2	Event-Triggered Periodic Reporting	1227

9.4A.2.4.3	Event-Triggered Reporting	1227

9.4A.3	NR − E-UTRAN TDD measurements	1228

9.4A.3.1	Introduction	1228

9.4A.3.2	Requirements when no DRX is used	1228

9.4A.3.3	Requirements when DRX is used	1230

9.4A.3.4	Measurement reporting requirements	1231

9.4A.3.4.1	Periodic Reporting	1231

9.4A.3.4.2	Event-Triggered Periodic Reporting	1231

9.4A.3.4.3	Event-Triggered Reporting	1231

9.4A.4	NR – E-UTRAN measurements with autonomous gaps	1232

9.4A.4.1	CGI identification of an E-UTRA cell with autonomous gaps	1232

9.4A.4.2	CGI reporting delay	1232

9.4A.4.3	CGI reporting scheduling restriction	1232

9.5	L1-RSRP measurements for Reporting	1233

9.5.1	Introduction	1233

9.5.2	Requirements applicability	1234

9.5.3	Measurement Reporting Requirements	1234

9.5.3.1	Periodic Reporting	1235

9.5.3.2	Semi-Persistent Reporting	1235

9.5.3.3	Aperiodic Reporting	1235

9.5.3.4	Event Triggered Reporting for the UE initiated beam management	1235

9.5.4	L1-RSRP measurement requirements	1237

9.5.4.1	SSB based L1-RSRP Reporting	1237

9.5.4.2	CSI-RS based L1-RSRP Reporting	1243

9.5.4A	Void	1248

9.5.4A.1	Void	1248

9.5.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	1248

9.5.5.1	Measurement restriction for SSB based L1-RSRP	1248

9.5.5.2	Measurement restriction for CSI-RS based L1-RSRP	1249

9.5.6	Scheduling availability of UE during L1-RSRP measurement	1250

9.5.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1251

9.5.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1251

9.5.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	1252

9.5.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	1253

9.5.7	Minimum requirement at transitions	1254

9.5A	L1-RSRP measurements for Reporting under CCA	1254

9.5A.1	Introduction	1254

9.5A.2	Requirements applicability	1254

9.5A.3	Measurement Reporting Requirements	1255

9.5A.3.1	Periodic Reporting	1255

9.5A.3.2	Semi-Persistent Reporting	1255

9.5A.3.3	Aperiodic Reporting	1255

9.5A.4	L1-RSRP measurement requirements	1256

9.5A.4.1	SSB based L1-RSRP Reporting	1256

9.5A.5	Measurement restriction for L1-RSRP measurement	1258

9.5A.5.1	Measurement restriction for SSB based L1-RSRP	1258

9.5A.6	Scheduling availability of UE during L1-RSRP measurement	1259

9.5A.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1259

9.5A.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1259

9.5A.6.3	Void	1259

9.5A.6.3A	Scheduling availability of UE performing L1-RSRP measurement in case of FR1-FR2 inter-band CA	1259

9.5A.6.3B	Scheduling availability of UE performing L1-RSRP measurement on FR2-2	1259

9.5A.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	1260

9.5B	L1-RSRP measurements for Reporting for RedCap	1260

9.5B.1	Introduction	1260

9.5B.2	Requirements applicability	1260

9.5B.3	Measurement Reporting Requirements	1261

9.5B.3.1	Periodic Reporting	1261

9.5B.3.2	Semi-Persistent Reporting	1261

9.5B.3.3	Aperiodic Reporting	1262

9.5B.4	L1-RSRP measurement requirements	1262

9.5B.4.1	SSB based L1-RSRP Reporting	1262

9.5B.4.2	CSI-RS based L1-RSRP Reporting	1264

9.5B.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	1267

9.5B.5.1	Measurement restriction for SSB based L1-RSRP	1267

9.5B.5.2	Measurement restriction for CSI-RS based L1-RSRP	1267

9.5B.6	Scheduling availability of UE during L1-RSRP measurement	1267

9.5B.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1268

9.5B.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1268

9.5B.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	1268

9.5C	L1-RSRP measurements for Reporting for satellite access	1269

9.5C.1	Introduction	1269

9.5C.3	Measurement Reporting Requirements	1269

9.5C.3.1	Periodic Reporting	1270

9.5C.3.2	Semi-Persistent Reporting	1270

9.5C.3.3	Aperiodic Reporting	1270

9.5C.4	L1-RSRP measurement requirements	1270

9.5C.4.1	SSB based L1-RSRP Reporting	1270

9.5C.5	Measurement restriction for L1-RSRP measurement	1272

9.5C.5.1	Measurement restriction for SSB based L1-RSRP	1272

9.5C.5.2	Measurement restriction for CSI-RS based L1-RSRP	1272

9.5C.6	Scheduling availability of UE during L1-RSRP measurement	1273

9.5C.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1-NTN	1273

9.5C.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1-NTN	1273

9.5C.7	L1-RSRP measurement requirements for NTN band above 10 GHz	1273

9.5C.7.1	SSB based L1-RSRP Reporting	1273

9.5C.7.2	CSI-RS based L1-RSRP Reporting	1274

9.5C.8	Measurement restriction for L1-RSRP measurement for NTN band above 10 GHz	1275

9.5C.8.1	Measurement restriction for SSB based L1-RSRP	1275

9.5C.8.2	Measurement restriction for CSI-RS based L1-RSRP	1275

9.5C.9	Scheduling availability of UE during L1-RSRP measurement for NTN band above 10 GHz	1276

9.5C.9.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on NTN bands above 10 GHz	1276

9.5C.9.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on NTN bands above 10 GHz	1276

9.5D	L1-RSRP measurements for Reporting for ATG	1276

9.5D.1	Introduction	1276

9.5D.2	Requirements applicability	1276

9.5D.3	Measurement Reporting Requirements	1277

9.5D.3.1	Periodic Reporting	1277

9.5D.3.2	Semi-Persistent Reporting	1277

9.5D.3.3	Aperiodic Reporting	1277

9.5D.4	L1-RSRP measurement requirements	1277

9.5D.4.1	SSB based L1-RSRP Reporting	1277

9.5D.4.2	CSI-RS based L1-RSRP Reporting	1279

9.5D.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	1281

9.5D.5.1	Measurement restriction for SSB based L1-RSRP	1281

9.5D.5.2	Measurement restriction for CSI-RS based L1-RSRP	1281

9.5D.6	Scheduling availability of UE during L1-RSRP measurement	1282

9.5D.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1282

9.5D.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1282

9.5E	L1-RSRP measurements for Reporting for RedCap UEs with satellite access	1282

9.5E.1	Introduction	1282

9.5E.2	Requirements applicability	1282

9.5E.3	Measurement Reporting Requirements	1283

9.5E.3.1	Periodic Reporting	1283

9.5E.3.2	Semi-Persistent Reporting	1283

9.5E.3.3	Aperiodic Reporting	1284

9.5E.4	L1-RSRP measurement requirements	1284

9.5E.4.1	SSB based L1-RSRP Reporting	1284

9.5E.4.2	CSI-RS based L1-RSRP Reporting	1284

9.5E.5	Measurement restriction for L1-RSRP measurement	1284

9.5E.6	Scheduling availability of UE during L1-RSRP measurement	1284

9.5E.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1284

9.5E.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1285

9.5F	L1-RSRP measurements Reporting for Beam Prediction	1285

9.5F.1	Introduction	1285

9.5F.2	Requirements applicability	1285

9.5F.3	Measurement Reporting Requirements	1286

9.5F.3.1	Periodic Reporting	1286

9.5F.3.2	Semi-Persistent Reporting	1286

9.5F.3.3	Aperiodic Reporting	1286

9.5F.4	L1-RSRP measurement and prediction requirements	1287

9.5F.4.1	SSB based RS prediction reporting	1287

9.5F.4.2	CSI-RS based RS prediction reporting	1288

9.6	NE-DC: Measurements	1290

9.6.1	Introduction	1290

9.6.2	SFTD Measurements	1291

9.6.2.1	Introduction	1291

9.6.2.2	SFTD Measurement requirements	1291

9.7	Cross Link Interference measurements	1291

9.7.1	Introduction	1291

9.7.2	SRS-RSRP measurements	1292

9.7.2.1	Introduction	1292

9.7.2.2	Requirements applicability	1292

9.7.2.3	Measurement Reporting Requirements	1292

9.7.2.3.1	Periodic Reporting	1292

9.7.2.3.2	Event-triggered Periodic Reporting	1292

9.7.2.3.3	Event Triggered Reporting	1292

9.7.2.4	Measurement capability	1293

9.7.2.5	SRS-RSRP measurement period	1293

9.7.3	CLI-RSSI measurements	1293

9.7.3.1	Introduction	1293

9.7.3.2	Requirements applicability	1293

9.7.3.3	Measurement Reporting Requirements	1293

9.7.3.3.1	Periodic Reporting	1294

9.7.3.3.2	Event-triggered Periodic Reporting	1294

9.7.3.3.3	Event Triggered Reporting	1294

9.7.3.4	Measurement capability	1294

9.7.3.5	CLI-RSSI measurement period	1294

9.7.4	Scheduling availability of UE during CLI measurements	1294

9.7.4.1	Scheduling availability of UE performing measurement on FR1	1294

9.7.4.2	Scheduling availability of UE performing measurement on FR2	1295

9.8	L1-SINR measurements for Reporting	1296

9.8.1	Introduction	1296

9.8.2	Requirements applicability	1297

9.8.3	Measurement Reporting Requirements	1298

9.8.3.1	Periodic Reporting	1298

9.8.3.2	Semi-Persistent Reporting	1298

9.8.4	L1-SINR measurement requirements	1298

9.8.4.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	1298

9.8.4.2	L1-SINR reporting with SSB based CMR and dedicated IMR configured	1303

9.8.4.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	1305

9.8.5	Measurement restriction for L1-SINR measurement	1308

9.8.5.1	Measurement restriction if SSB configured for L1-SINR Measurement	1308

9.8.5.2	Measurement restriction if CSI-RS configured for L1-SINR measurement	1309

9.8.5.3	Measurement restriction if CSI-IM configured for L1-SINR measurement	1310

9.8.6	Scheduling availability of UE during L1-SINR measurement	1311

9.8.6.1	Scheduling availability of UE performing L1-SINR measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1311

9.8.6.2	Scheduling availability of UE performing L1-SINR measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1311

9.8.6.4	Scheduling availability of UE performing L1-SINR measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	1313

9.8.7	Minimum requirement at transitions	1313

9.8D	L1-SINR measurements for Reporting for ATG	1313

9.8D.1	Introduction	1313

9.8D.2	Requirements applicability	1314

9.8D.3	Measurement Reporting Requirements	1314

9.8D.3.1	Periodic Reporting	1315

9.8D.3.2	Semi-Persistent Reporting	1315

9.8D.3.3	Aperiodic Reporting	1315

9.8D.4	L1-SINR measurement requirements	1315

9.8D.4.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	1315

9.8D.4.2	L1-SINR reporting with SSB based CMR and dedicated IMR configured	1317

9.8D.4.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	1318

9.8D.5	Measurement restriction for L1-SINR measurement	1319

9.8D.5.1	Measurement restriction if SSB configured for L1-SINR Measurement	1319

9.8D.5.2	Measurement restriction if CSI-RS configured for L1-SINR measurement	1319

9.8D.5.3	Measurement restriction if CSI-IM configured for L1-SINR measurement	1320

9.8D.6	Scheduling availability of UE during L1-SINR measurement	1320

9.8D.6.1	Scheduling availability of UE performing L1-SINR measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1320

9.8D.6.2	Scheduling availability of UE performing L1-SINR measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1320

9.9	NR measurements for positioning	1320

9.9.1	Introduction	1320

9.9.1.1	General Aspects of Gap-based Measurement	1320

9.9.1.2	General Aspects of Gapless Measurement	1321

9.9.1.3	Scheduling Availability of UE during PRS Measurement without Measurement Gaps	1322

9.9.2	RSTD measurements	1323

9.9.2.1	Introduction	1323

9.9.2.2	Requirements Applicability	1323

9.9.2.3	Measurement Capability	1324

9.9.2.4	Measurement Reporting Requirements	1324

9.9.2.4.1	Void	1324

9.9.2.4.2	Void	1324

9.9.2.4.3	Void	1324

9.9.2.5	Measurements Period Requirements	1324

9.9.2.6	Void	1327

9.9.2.7	Measurements Period Requirements without Measurement Gaps	1327

9.9.2.8	Void	1330

9.9.2.9	Measurements Period Requirements with both MG and PPW	1330

9.9.2.10	Measurements Period Requirements with Bandwidth Aggregation	1331

9.9.3	PRS-RSRP measurements	1334

9.9.3.1	Introduction	1334

9.9.3.2	Requirements applicability	1334

9.9.3.3	Measurement Capability	1334

9.9.3.4	Measurement Reporting Requirements	1335

9.9.3.5	Measurement Period Requirements	1335

9.9.3.6	Measurement Period Requirements without Measurement Gaps	1337

9.9.3.7	Void	1340

9.9.3.8	Measurements Period Requirements with both MG and PPW	1340

9.9.4	UE Rx-Tx time difference measurements	1340

9.9.4.1	Introduction	1340

9.9.4.2	Requirements Applicability	1341

9.9.4.3	Measurement Capability	1341

9.9.4.4	Measurement Reporting Requirements	1341

9.9.4.5	Measurement Period Requirements	1341

9.9.4.6	Measurement Period Requirements without Measurement Gaps	1345

9.9.4.7	Void	1348

9.9.4.8	Measurements Period Requirements with both MG and PPW	1348

9.9.4.9	Measurements Period Requirements with Bandwidth Aggregation	1348

9.9.5	E-CID measurements	1353

9.9.5.1	Introduction	1353

9.9.5.2	Measurement Requirements	1353

9.9.5.2.1	Intra-frequency Measurement Requirements	1353

9.9.5.2.2	Inter-frequency Measurement Requirements	1353

9.9.5.2.3	Measurement Reporting Delay	1353

9.9.6	PRS-RSRPP measurements	1354

9.9.6.1	Introduction	1354

9.9.6.2	Requirements applicability	1354

9.9.6.3	Measurement capability	1354

9.9.6.4	Measurement reporting requirements	1354

9.9.6.5	Measurement period requirements	1354

9.9.6.6	Measurement Period Requirements without Measurement Gaps	1354

9.9.6.7	Void	1355

9.9.6.8	Measurements Period Requirements with both MG and PPW	1355

9.9.7	Measurement requirements for DL RSCPD reported with RSTD	1355

9.9.7.1	Introduction	1355

9.9.7.2	Requirements Applicability	1355

9.9.7.3	Measurement Capability	1355

9.9.7.4	Measurement Reporting Requirements	1355

9.9.7.5	Measurements Period Requirements for DL RSCPD reported with RSTD	1355

9.9.8	Measurement requirements for DL RSCP reported with UE Rx-Tx time difference	1358

9.9.8.1	Introduction	1358

9.9.8.2	Requirements Applicability	1358

9.9.8.3	Measurement Capability	1359

9.9.8.4	Measurement Reporting Requirements	1359

9.9.8.5	Measurement Period Requirements for DL RSCP and UE Rx-Tx time difference	1359

9.9A	NR measurements for positioning for RedCap	1363

9.9A.1	Introduction	1363

9.9A.1.1	General Aspects of Gap-based Measurement	1363

9.9A.1.2	General Aspects of Gapless Measurement for RedCap positioning without FH	1364

9.9A.1.3	Scheduling Availability of UE during PRS Measurement without Measurement Gaps for RedCap positioning without FH	1365

9.9A.2	RSTD measurements for RedCap	1366

9.9A.2.1	Introduction	1366

9.9A.2.2	Requirements Applicability	1366

9.9A.2.3	Measurement Capability	1366

9.9A.2.4	Measurement Reporting Requirements	1366

9.9A.2.5	Measurements Period Requirements without FH	1366

9.9A.2.5.1	Measurements Period Requirements without FH with MG	1366

9.9A.2.5.2	Measurements Period Requirements without FH without MG	1369

9.9A.2.5.3	Measurements Period Requirements without FH with both MG and PPW	1372

9.9A.2.6	Measurements Period Requirements with FH	1373

9.9A.2.6.1	Measurements Period Requirements with FH with MG	1373

9.9A.3	PRS-RSRP measurements for RedCap	1375

9.9A.3.1	Introduction	1375

9.9A.3.2	Requirements applicability	1375

9.9A.3.3	Measurement Capability	1375

9.9A.3.4	Measurement Reporting Requirements	1375

9.9A.3.5	Measurements Period Requirements without FH	1375

9.9A.3.5.1	Measurement Period Requirements without FH with MG	1375

9.9A.3.5.2	Measurement Period Requirements without FH without MG	1378

9.9A.3.5.3	Measurements Period Requirements without FH with both MG and PPW	1380

9.9A.3.6	Measurements Period Requirements with FH	1381

9.9A.3.6.1	Measurements Period Requirements with FH with MG	1381

9.9A.4	UE Rx-Tx time difference measurements for RedCap	1383

9.9A.4.1	Introduction	1383

9.9A.4.2	Requirements Applicability	1383

9.9A.4.3	Measurement Capability	1383

9.9A.4.4	Measurement Reporting Requirements	1383

9.9A.4.5	Measurement Period Requirements without FH with MG	1383

9.9A.4.6	Measurement Period Requirements without FH without MG	1383

9.9A.4.7	Measurements Period Requirements without FH with both MG and PPW	1383

9.9A.4.8	Measurements Period Requirements with FH	1383

9.9A.5	PRS-RSRPP measurements for RedCap	1385

9.9A.5.1	Introduction	1385

9.9A.5.2	Requirements Applicability	1385

9.9A.5.3	Measurement Capability	1385

9.9A.5.4	Measurement Reporting Requirements	1386

9.9A.5.5	Measurement Period Requirements without FH with MG	1386

9.9A.5.6	Measurement Period Requirements without FH without MG	1386

9.9A.5.7	Measurements Period Requirements without FH with both MG and PPW	1386

9.9A.5.8	Measurements Period Requirements with FH	1386

9.9C	NR measurements for positioning in Satellite Access	1386

9.9C.1	Introduction	1386

9.9C.1.1	General Aspects of Gap-based Measurement	1386

9.9C.1.2	General Aspects of Gapless Measurement	1387

9.9C.1.3	Scheduling Availability of UE during PRS Measurement without Measurement Gaps	1388

9.9C.2	Void	1388

9.9C.3	Void	1388

9.9C.4	UE Rx-Tx time difference measurements	1388

9.9C.4.1 Introduction	1388

9.9C.4.2 Requirements Applicability	1388

9.9C.4.3	Measurement Capability	1388

9.9C.4.4	Measurement Reporting Requirements	1388

9.9C.4.5	Measurement Period Requirements	1388

9.9C.4.6	Measurement Period Requirements without Measurement Gaps	1391

9.9D	NR measurements for positioning for RedCap in Satellite Access	1393

9.9D.1	Introduction	1393

9.9D.1.1	General Aspects of Gap-based Measurement	1393

9.9D.1.2	General Aspects of Gapless Measurement	1393

9.9D.1.3	Scheduling Availability of UE during PRS Measurement without Measurement Gaps	1393

9.9D.2	Void	1394

9.9D.3	Void	1394

9.9D.4	UE Rx-Tx time difference measurements	1394

9.9D.4.1 Introduction	1394

9.9D.4.2 Requirements Applicability	1394

9.9D.4.3	Measurement Capability	1394

9.9D.4.4	Measurement Reporting Requirements	1394

9.9D.4.5	Measurement Period Requirements	1394

9.9D.4.6	Measurement Period Requirements without Measurement Gaps	1395

9.9E	Reporting Delay Requirements for DL AI/ML Positioning	1395

9.9E.1	Introduction	1395

9.9E.2	General Aspects Relating to Gap-based Measurement	1395

9.9E.3	General Aspects Relating to Gapless Measurement	1396

9.9E.4	Scheduling Availability Relating to Gapless Measurement	1397

9.9E.5	Measurement Delay Requirement with Measurement Gaps	1397

9.9E.6	Measurement Delay Requirement without Measurement Gaps	1399

9.9E.7	Measurement Delay Requirement with Bandwidth Aggregation	1402

9.10	CSI-RS based L3 measurements	1405

9.10.1	Introduction	1405

9.10.2	CSI-RS based intra-frequency measurements	1405

9.10.2.1	Introduction	1405

9.10.2.2	Requirements applicability	1406

9.10.2.3	Number of cells and number of CSI-RS	1407

9.10.2.3.1	Requirements for FR1	1407

9.10.2.3.2	Requirements for FR2	1407

9.10.2.4	Measurement Reporting Requirements	1407

9.10.2.4.1	Periodic Reporting	1407

9.10.2.4.2	Event-triggered Periodic Reporting	1408

9.10.2.4.3	Event Triggered Reporting	1408

9.10.2.5	Intra-frequency measurements without measurement gaps	1408

9.10.2.6	Scheduling availability of UE during CSI-RS based intra-frequency measurements	1410

9.10.2.6.1	Scheduling availability of UE performing CSI-RS based measurements in TDD bands	1410

9.10.2.6.2	Scheduling availability of UE performing CSI-RS based measurements in FR2	1411

9.10.3	CSI-RS based Inter-frequency measurements	1411

9.10.3.1	Introduction	1411

9.10.3.2	Requirements applicability	1411

9.10.3.3	Number of cells and number of CSI-RS resources	1412

9.10.3.3.1	Requirements for FR1	1412

9.10.3.3.2	Requirements for FR2	1412

9.10.3.4	Measurements reporting requirements	1412

9.10.3.4.1	Periodic Reporting	1412

9.10.3.4.2	Event-triggered Periodic Reporting	1412

9.10.3.4.3	Event-triggered Reporting	1413

9.10.3.5	Inter-frequency measurements with measurement gaps	1413

9.10D	CSI-RS based L3 measurements for ATG	1415

9.10D.1	Introduction	1415

9.10D.2	CSI-RS based intra-frequency measurements	1415

9.10D.2.1	Introduction	1415

9.10D.2.2	Requirements applicability	1415

9.10D.2.3	Number of cells and number of CSI-RS	1416

9.10D.2.3.1	Requirements for FR1	1416

9.10D.2.4	Measurement Reporting Requirements	1416

9.10D.2.4.1	Periodic Reporting	1417

9.10D.2.4.2	Event-triggered Periodic Reporting	1417

9.10D.2.4.3	Event Triggered Reporting	1417

9.10D.2.5	Intra-frequency measurements without measurement gaps	1417

9.10D.2.6	Scheduling availability of UE during CSI-RS based intra-frequency measurements	1419

9.10D.2.6.1	Scheduling availability of UE performing CSI-RS based measurements in TDD bands	1420

9.10D.3	CSI-RS based Inter-frequency measurements	1420

9.10D.3.1	Introduction	1420

9.10D.3.2	Requirements applicability	1420

9.10D.3.3	Number of cells and number of CSI-RS resources	1421

9.10D.3.3.1	Requirements for FR1	1421

9.10D.3.4	Measurements reporting requirements	1421

9.10D.3.4.1	Periodic Reporting	1421

9.10D.3.4.2	Event-triggered Periodic Reporting	1421

9.10D.3.4.3	Event-triggered Reporting	1421

9.10D.3.5	Inter-frequency measurements with measurement gaps	1422

9.11	NR measurements with autonomous gaps	1423

9.11.1	Introduction	1423

9.11.2	CGI identification of an NR cell with autonomous gaps	1423

9.11.3	CGI reporting delay	1424

9.11A	NR measurements with autonomous gaps for RedCap	1424

9.11A.1	Introduction	1424

9.11A.2	CGI identification of an NR cell with autonomous gaps	1425

9.11A.3	CGI reporting delay	1425

9.11A.4	CGI reporting scheduling restriction	1426

9.11D	NR measurements with autonomous gaps for ATG	1426

9.11D.1	Introduction	1426

9.11D.2	CGI identification of an NR cell with autonomous gaps	1427

9.11D.3	CGI reporting delay	1427

9.12	Measurement for Propagation Delay Compensation	1428

9.12.1	Introduction	1428

9.12.2	Requirements Applicability	1428

9.12.3	Measurement Capability	1428

9.12.4	Measurement period requirements	1428

9.12.4.1	PRS Measurement Period	1428

9.12.4.2	TRS Measurement Period	1429

9.12.5	Measurement Reporting Requirements	1430

9.12.6	Scheduling availability during measurement for Propagation Delay Compensation	1430

9.12.7	Measurement restriction for measurement for Propagation Delay Compensation	1430

9.12.8	Measurement requirement for Propagation Delay Compensation with MUSIM gaps	1431

9.13	L1-RSRP measurements for a cell with different PCI from serving cell	1431

9.13.1	Introduction	1431

9.13.2	Requirements Applicability	1431

9.13.3	Measurement Reporting Requirements	1432

9.13.3.1	Periodic Reporting	1432

9.13.3.2	Semi-Persistent Reporting	1432

9.13.3.3	Aperiodic Reporting	1432

9.13.3.4	Event triggered reporting for UE initiated beam management	1433

9.13.4	L1-RSRP measurement requirements	1434

9.13.4.1	Inter-cell SSB based L1-RSRP Reporting	1434

9.13.5	Measurement restriction for L1-RSRP measurement	1436

9.13.5.1	Measurement restriction for SSB based L1-RSRP	1437

9.13.6	Scheduling availability of UE during L1-RSRP measurement	1438

9.13.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1438

9.13.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1438

9.13.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	1439

9.13.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	1439

9.13.6.5	Scheduling availability of UE performing L1-RSRP measurement in TDD bands on FR1	1440

9.14	NR intra-frequency L1-RSRP measurements for neighbor cell	1440

9.14.1	Introduction	1440

9.14.2	Requirements Applicability	1440

9.14.3	Measurement Reporting Requirements	1441

9.14.3.1	Periodic Reporting	1441

9.14.3.2	Semi-Persistent Reporting	1441

9.14.3.3	Aperiodic Reporting	1441

9.14.3.4	Event Triggered Reporting	1441

9.14.3.5	Event-triggered Periodic Reporting	1442

9.14.4	Number of SSB frequency layers, number of cells and number of SSBs	1442

9.14.5	L1-RSRP intra-frequency measurement requirements without measurement gaps	1442

9.14.5.1	SSB based L1-RSRP reporting	1442

9.14.6	Measurement restriction for L1-RSRP measurement	1445

9.14.6.1	Measurement restriction for SSB based L1-RSRP	1445

9.14.7	Scheduling availability of UE during L1-RSRP measurement	1446

9.14.7.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1446

9.14.7.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1446

9.14.7.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	1447

9.14.7.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	1447

9.14.7.5	Scheduling availability of UE performing L1-RSRP measurement in TDD bands on FR1	1447

9.14a	CSI-RS based Intra-frequency L1-RSRP measurements for neighbour cell	1448

9.14a.1	Introduction	1448

9.14a.2	Requirements Applicability	1448

9.14a.3	Measurement Reporting Requirements	1449

9.14a.3.1	Periodic Reporting	1449

9.14a.3.2	Semi-Persistent Reporting	1449

9.14a.3.3	Aperiodic Reporting	1449

9.14a.3.4	Event-triggered Periodic Reporting	1449

9.14a.3.5	Event Triggered Reporting	1449

9.14a.4	Number of CSI-RS resources, number of cells	1450

9.14a.5	CSI-RS based L1-RSRP measurement requirements without measurement gaps	1450

9.14a.6	Measurement restriction for CSI-RS based L1-RSRP measurement	1452

9.14a.6.1	Measurement restriction for CSI-RS based L1-RSRP measurement	1452

9.14a.7	Scheduling availability of UE during CSI-RS based L1-RSRP measurement	1453

9.14a.7.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1453

9.14a.7.2	Scheduling availability of UE performing L1-RSRP measurement on FR2	1453

9.14a.7.3	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	1454

9.14a.7.4	Scheduling availability of UE performing L1-RSRP measurement in TDD bands on FR1	1454

9.15	NR inter-frequency L1-RSRP measurements for neighbor cell	1454

9.15.1	Introduction	1454

9.15.2	Requirements Applicability	1454

9.15.3	Measurement Reporting Requirements	1455

9.15.3.1	Periodic Reporting	1455

9.15.3.2	Semi-Persistent Reporting	1455

9.15.3.3	Aperiodic Reporting	1456

9.15.3.4	Event Triggered Reporting	1456

9.15.3.5	Event Triggered Periodic Reporting	1456

9.15.4	Number of SSB frequency layers, number of cells and number of SSBs	1456

9.15.5	L1-RSRP inter-frequency measurement requirements with measurement gaps	1456

9.15.5.1	Inter-frequency SSB based L1-RSRP reporting	1456

9.15.6	L1-RSRP inter-frequency L1-RSRP measurement requirements without measurement gaps	1459

9.15.6.1	Inter-frequency L1-RSRP measurement requirements	1459

9.15.6.1.1	Inter-frequency SSB based L1-RSRP measurement	1459

9.15.6.2	Measurement restriction for inter-frequency L1-RSRP measurement	1462

9.15.6.2.1	Measurement restriction for SSB based L1-RSRP	1462

9.15.6.3	Scheduling availability of UE during inter-frequency L1-RSRP measurements	1462

9.15.6.3.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1462

9.15.6.3.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1463

9.15.6.3.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	1463

9.15.6.3.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	1463

9.15.6.3.5	Scheduling availability of UE performing L1-RSRP measurement in TDD bands on FR1	1464

9.16	CJT calibration reporting for Delay offset and Frequency offset	1464

9.16.1	Introduction	1464

9.16.2	Requirements applicability	1464

9.16.3	Measurement Reporting Requirements	1464

9.16.3.1	Aperiodic Reporting	1464

After the UE receives CSI request in DCI with (reportQuantity set to ‘cjtc-Dd’, ‘cjtc-F’), the UE shall transmit the aperiodic CJTC reporting on PUSCH over the air interface at the time specified according to relevant clause in [26].9.16.4 Measurement Requirements	1465

9.16.4.1 CSI -RS based delay and frequency offset reporting	1465

9.16.5	Measurement restriction for UE during CJT calibration reporting	1466

9.16.5.1	Measurement restriction for CJT calibration reporting	1466

9.16.6	Scheduling availability of UE during CJT calibration reporting	1467

9.16.6.1	Scheduling availability of UE performing measurement for CJT calibration reporting on FR1	1467

9.17	OD-SSB based L3 measurement for an SCell	1467

9.17.1	Introduction	1467

9.17.2	Requirements Applicability	1468

9.17.3	Number of cells and number of SSB	1468

9.17.4	Measurement Reporting Requirements	1468

9.17.5	OD-SSB based Intra-frequency measurements without measurement gaps	1468

9.17.5.1	Intra-frequency cell identification for active SCell	1468

9.17.5.2	Measurement period for active SCell	1473

9.17.5.3	Intra-frequency cell identification for deactivated SCell	1475

9.17.5.4	Measurement period for deactivated SCell	1478

9.17.5.5	Scheduling availability of UE during intra-frequency measurements based on On-demand SSB	1479

9.17.5.5.1	Scheduling availability of UE performing measurements in TDD bands on FR1	1479

9.17.5.5.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	1479

9.17.5.5.3	Scheduling availability of UE performing measurements on FR2-1	1480

9.17.5.5.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	1481

9.18	L1 Cross Link Interference measurements	1481

9.18.1	Introduction	1481

9.18.2	L1-SRS-RSRP measurements	1482

9.18.2.1	Introduction	1482

9.18.2.2	Requirements applicability	1482

9.18.2.3	Measurement Reporting Requirements	1482

9.18.2.3.1	Aperiodic Reporting	1482

9.18.2.4	Measurement capability	1483

9.18.2.5	L1-SRS-RSRP measurement period	1483

9.18.2.6	Scheduling availability of UE during L1-CLI measurements	1483

9.18.2.6.1	Scheduling availability of UE performing L1-SRS-RSRP measurement on FR1	1483

9.18.2.6.2	Scheduling availability of UE performing L1-SRS-RSRP measurement on FR2	1484

9.18.3	L1-CLI-RSSI measurements	1485

9.18.3.1	Introduction	1485

9.18.3.2	Requirements applicability	1485

9.18.3.3	Measurement Reporting Requirements	1485

9.18.3.3.1	Periodic Reporting	1485

9.18.3.3.2	Aperiodic Reporting	1485

9.18.3.4	Measurement capability	1486

9.18.3.5	L1-CLI-RSSI measurement period	1486

9.18.3.6	Scheduling availability of UE during L1-CLI-RSSI measurements	1486

9.18.3.6.1	Scheduling availability of UE performing L1-CLI-RSSI measurement on FR1	1486

9.18.3.6.2	Scheduling availability of UE performing L1-CLI-RSSI measurement on FR2	1487

10	Measurement Performance requirements	1489

10.1	NR measurements	1489

10.1.1	Introduction	1489

10.1.2	Intra-frequency RSRP accuracy requirements for FR1	1489

10.1.2.1	Intra-frequency SS-RSRP accuracy requirements	1489

10.1.2.1.1	Absolute SS-RSRP Accuracy	1489

10.1.2.1.2	Relative SS-RSRP Accuracy	1490

10.1.2.2	Void	1491

10.1.2.3	Intra-frequency CSI-RSRP accuracy requirements	1491

10.1.2.3.1	Absolute CSI-RSRP Accuracy	1491

10.1.2.3.2	Relative CSI-RSRP Accuracy	1492

10.1.2B	Intra-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	1493

10.1.2B.1	Intra-frequency SS-RSRP accuracy requirements	1493

10.1.2B.1.1	Absolute SS-RSRP Accuracy	1493

10.1.2C	Intra-frequency RSRP accuracy requirements for FR1 SAN	1494

10.1.2C.1	Intra-frequency SS-RSRP accuracy requirements	1494

10.1.2C.1.1	Absolute SS-RSRP Accuracy	1494

10.1.2C.1.2	Relative SS-RSRP Accuracy	1495

10.1.3	Intra-frequency RSRP accuracy requirements for FR2	1496

10.1.3.1	Intra-frequency SS-RSRP accuracy requirements	1496

10.1.3.1.1	Absolute SS-RSRP Accuracy	1496

10.1.3.1.2	Relative SS-RSRP Accuracy	1496

10.1.3.2	Void	1497

10.1.3.3	Intra-frequency CSI-RSRP accuracy requirements	1497

10.1.3.3.1	Absolute CSI-RSRP Accuracy	1497

10.1.3.3.2	Relative CSI-RSRP Accuracy	1498

10.1.3B	Intra-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	1499

10.1.3B.1	Intra-frequency SS-RSRP accuracy requirements	1499

10.1.3B.1.1	Absolute SS-RSRP Accuracy	1499

10.1.3C	Intra-frequency RSRP accuracy requirements for FR2-NTN	1499

10.1.3C.1	Intra-frequency SS-RSRP accuracy requirements	1499

10.1.3C.1.1	Absolute SS-RSRP Accuracy	1499

10.1.3C.1.2	Relative SS-RSRP Accuracy	1500

10.1.4	Inter-frequency RSRP accuracy requirements for FR1	1501

10.1.4.1	Inter-frequency SS-RSRP accuracy requirements	1501

10.1.4.1.1	Absolute SS-RSRP Accuracy in FR1	1501

10.1.4.1.2	Relative SS-RSRP Accuracy in FR1	1502

10.1.4.2	Void	1503

10.1.4.3	Inter-frequency CSI-RSRP accuracy requirements	1503

10.1.4.3.1	Absolute CSI-RSRP Accuracy in FR1	1503

10.1.4.3.2	Relative CSI-RSRP Accuracy in FR1	1504

10.1.4B	Inter-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	1505

10.1.4B.1	Inter-frequency SS-RSRP accuracy requirements	1505

10.1.4B.1.1	Absolute SS-RSRP Accuracy in FR1	1506

10.1.4C	Inter-frequency RSRP accuracy requirements for FR1 SAN	1507

10.1.4C.1	Inter-frequency SS-RSRP accuracy requirements	1507

10.1.4C.1.1	Absolute SS-RSRP Accuracy in FR1	1507

10.1.4C.1.2	Relative SS-RSRP Accuracy in FR1	1507

10.1.5	Inter-frequency RSRP accuracy requirements for FR2	1508

10.1.5.1	Inter-frequency SS-RSRP accuracy requirements	1508

10.1.5.1.1	Absolute SS-RSRP Accuracy	1508

10.1.5.1.2	Relative SS-RSRP Accuracy	1508

10.1.5.2	Void	1509

10.1.5.3	Inter-frequency CSI-RSRP accuracy requirements	1509

10.1.5.3.1	Absolute CSI-RSRP Accuracy	1509

10.1.5.3.2	Relative CSI-RSRP Accuracy	1510

10.1.5B	Inter-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	1511

10.1.5B.1	Inter-frequency SS-RSRP accuracy requirements	1511

10.1.5B.1.1	Absolute SS-RSRP Accuracy	1511

10.1.5C	Inter-frequency RSRP accuracy requirements for FR2-NTN	1512

10.1.5C.1	Inter-frequency SS-RSRP accuracy requirements	1512

10.1.5C.1.1	Absolute SS-RSRP Accuracy	1512

10.1.5C.1.2	Relative SS-RSRP Accuracy	1512

10.1.6	RSRP Measurement Report Mapping	1513

10.1.7	Intra-frequency RSRQ accuracy requirements for FR1	1515

10.1.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	1515

10.1.7.1.1	Absolute SS-RSRQ Accuracy in FR1	1515

10.1.7.2	Intra-frequency CSI-RSRQ accuracy requirements	1516

10.1.7.2.1	Absolute CSI-RSRQ Accuracy	1516

10.1.7B	Intra-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	1517

10.1.7B.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	1517

10.1.7B.1.1	Absolute SS-RSRQ Accuracy in FR1	1517

10.1.7C	Intra-frequency RSRQ accuracy requirements for FR1 SAN	1518

10.1.7C.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	1518

10.1.7C.1.1	Absolute SS-RSRQ Accuracy in FR1	1518

10.1.8	Intra-frequency RSRQ accuracy requirements for FR2	1519

10.1.8.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	1519

10.1.8.1.1	Absolute SS-RSRQ Accuracy in FR2	1519

10.1.8.2	Intra-frequency CSI-RSRQ accuracy requirements	1519

10.1.8.2.1	Absolute CSI-RSRQ Accuracy	1519

10.1.8B	Intra-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	1520

10.1.8B.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	1520

10.1.8B.1.1	Absolute SS-RSRQ Accuracy in FR2	1520

10.1.8C	Intra-frequency RSRQ accuracy requirements for FR2-NTN	1521

10.1.8C.1	Intra-frequency SS-RSRQ accuracy requirements in FR2-NTN	1521

10.1.8C.1.1	Absolute SS-RSRQ Accuracy in FR2-NTN	1521

10.1.9	Inter-frequency RSRQ accuracy requirements for FR1	1522

10.1.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	1522

10.1.9.1.1	Absolute SS-RSRQ Accuracy in FR1	1522

10.1.9.1.2	Relative SS-RSRQ Accuracy in FR1	1522

10.1.9.2	Inter-frequency CSI-RSRQ accuracy requirements	1523

10.1.9.2.1	Absolute CSI-RSRQ Accuracy	1523

10.1.9.2.2	Relative CSI-RSRQ Accuracy	1524

10.1.9B	Inter-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	1525

10.1.9B.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	1525

10.1.9B.1.1	Absolute SS-RSRQ Accuracy in FR1	1525

10.1.9C	Inter-frequency RSRQ accuracy requirements for FR1 SAN	1526

10.1.9C.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	1526

10.1.9C.1.1	Absolute SS-RSRQ Accuracy in FR1	1526

10.1.9C.1.2	Relative SS-RSRQ Accuracy in FR1	1527

10.1.10	Inter-frequency RSRQ accuracy requirements for FR2	1527

10.1.10.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	1527

10.1.10.1.1	Absolute SS-RSRQ Accuracy in FR2	1527

10.1.10.1.2	Relative SS-RSRQ Accuracy in FR2	1528

10.1.10.2	Inter-frequency CSI-RSRQ accuracy requirements	1529

10.1.10.2.1	Absolute CSI-RSRQ Accuracy	1529

10.1.10.2.2	Relative CSI-RSRQ Accuracy	1530

10.1.10B	 Inter-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	1531

10.1.10B.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	1531

10.1.10B.1.1	Absolute SS-RSRQ Accuracy in FR2	1531

10.1.10C	Inter-frequency RSRQ accuracy requirements for FR2-NTN	1532

10.1.10C.1	Inter-frequency SS-RSRQ accuracy requirements in FR2-NTN	1532

10.1.10C.1.1	Absolute SS-RSRQ Accuracy in FR2-NTN	1532

10.1.10C.1.2	Relative SS-RSRQ Accuracy in FR2-NTN	1532

10.1.11	RSRQ report mapping	1533

10.1.12	Intra-frequency SINR accuracy requirements for FR1	1534

10.1.12.1	Intra-frequency SS-SINR accuracy requirements in FR1	1534

10.1.12.1.1	Absolute SS-SINR Accuracy in FR1	1534

10.1.12.2	Intra-frequency CSI-SINR accuracy requirements in FR1	1534

10.1.12.2.1	Absolute CSI-SINR Accuracy in FR1	1534

10.1.12C	 Intra-frequency SINR accuracy requirements for FR1 SAN	1535

10.1.12C.1	Intra-frequency SS-SINR accuracy requirements in FR1	1535

10.1.12C.1.1	Absolute SS-SINR Accuracy in FR1	1535

10.1.13	Intra-frequency SINR accuracy requirements for FR2	1536

10.1.13.1	Intra-frequency SS-SINR accuracy requirements in FR2	1536

10.1.13.1.1	Absolute SS-SINR Accuracy in FR2	1536

10.1.13.2	Intra-frequency CSI-SINR accuracy requirements in FR2	1537

10.1.13.2.1	Absolute CSI-SINR Accuracy in FR2	1537

10.1.13C	Intra-frequency SINR accuracy requirements for FR2-NTN	1537

10.1.13C.1	Intra-frequency SS-SINR accuracy requirements in FR2-NTN	1537

10.1.13C.1.1	Absolute SS-SINR Accuracy in FR2-NTN	1537

10.1.14	Inter-frequency SINR accuracy requirements for FR1	1538

10.1.14.1	Inter-frequency SS-SINR accuracy requirements in FR1	1538

10.1.14.1.1	Absolute SS-SINR Accuracy in FR1	1538

10.1.14.1.2	Relative SS-SINR Accuracy in FR1	1539

10.1.14.2	Inter-frequency CSI-SINR accuracy requirements in FR1	1540

10.1.14.2.1	Absolute CSI-SINR Accuracy in FR1	1540

10.1.14.2.2	Relative CSI-SINR Accuracy in FR1	1541

10.1.14C	 Inter-frequency SINR accuracy requirements for FR1 SAN	1542

10.1.14C.1	Inter-frequency SS-SINR accuracy requirements in FR1	1542

10.1.14C.1.1	Absolute SS-SINR Accuracy in FR1	1542

10.1.14C.1.2	Relative SS-SINR Accuracy in FR1	1543

10.1.15	Inter-frequency SINR accuracy requirements for FR2	1544

10.1.15.1	Inter-frequency SS-SINR accuracy requirements in FR2	1544

10.1.15.1.1	Absolute SS-SINR Accuracy in FR2	1544

10.1.15.1.2	Relative SS-SINR Accuracy in FR2	1544

10.1.15.2	Inter-frequency CSI-SINR accuracy requirements in FR2	1545

10.1.15.2.1	Absolute CSI-SINR Accuracy in FR2	1545

10.1.15.2.2	Relative CSI-SINR Accuracy in FR2	1546

10.1.15C	Inter-frequency SINR accuracy requirements for FR2-NTN	1547

10.1.15C.1	Inter-frequency SS-SINR accuracy requirements in FR2-NTN	1547

10.1.15C.1.1	Absolute SS-SINR Accuracy in FR2-NTN	1547

10.1.15C.1.2	Relative SS-SINR Accuracy in FR2-NTN	1547

10.1.16	SINR report mapping	1548

10.1.16.1	SS-SINR and CSI-SINR measurement report mapping	1548

10.1.17	Power Headroom	1549

10.1.17.1	Power Headroom Report	1549

10.1.17.1.1	Power Headroom Report Mapping	1549

10.1.18	PCMAX,c,f	1549

10.1.18.1	Report Mapping	1549

10.1.19	L1-RSRP accuracy requirements for FR1	1550

10.1.19.1	SSB based L1-RSRP accuracy requirements	1550

10.1.19.1.1	Absolute Accuracy	1550

10.1.19.1.2	Relative Accuracy	1551

10.1.19.2	CSI-RS based L1-RSRP accuracy requirements	1552

10.1.19.2.1	Absolute Accuracy	1552

10.1.19.2.2	Relative Accuracy	1553

10.1.19C	L1-RSRP accuracy requirements for FR1 SAN	1554

10.1.19C.1	SSB based L1-RSRP accuracy requirements	1554

10.1.19C.1.1	Absolute Accuracy	1554

10.1.19C.1.2	Relative Accuracy	1555

10.1.19C.2	CSI-RS based L1-RSRP accuracy requirements	1555

10.1.19C.2.1	Absolute Accuracy	1555

10.1.19C.2.2	Relative Accuracy	1556

10.1.19D	LTM Intra-frequency L1-RSRP accuracy requirements for FR1	1557

10.1.19D.1	SSB based intra-frequency L1-RSRP accuracy requirements	1557

10.1.19D.1.1	Absolute Accuracy	1557

10.1.19D.1.2	Relative Accuracy	1558

10.1.19E	  LTM Inter-frequency L1-RSRP accuracy requirements for FR1	1558

10.1.19E.1	SSB based Inter-frequency L1-RSRP accuracy requirements	1558

10.1.19E.1.1	Absolute Accuracy	1558

10.1.19E.1.2	Relative Accuracy	1559

10.1.20	L1-RSRP accuracy requirements for FR2	1560

10.1.20.1	SSB based L1-RSRP accuracy requirements	1560

10.1.20.1.1	Absolute Accuracy	1560

10.1.20.1.2	Relative Accuracy	1561

10.1.20.2	CSI-RS based L1-RSRP accuracy requirements	1562

10.1.20.2.1	Absolute Accuracy	1562

10.1.20.2.2	Relative Accuracy	1562

10.1.20A	  LTM Intra-frequency L1-RSRP accuracy requirements for FR2	1563

10.1.20A.1	SSB based intra-frequency L1-RSRP accuracy requirements	1563

10.1.20A.1.1	Absolute Accuracy	1563

10.1.20A.1.2	Relative Accuracy	1564

10.1.20B	  LTM Inter-frequency L1-RSRP accuracy requirements for FR2	1565

10.1.20B.1	SSB based inter-frequency L1-RSRP accuracy requirements	1565

10.1.20B.1.1	Absolute Accuracy	1565

10.1.20B.1.2	Relative Accuracy	1565

10.1.20C	L1-RSRP accuracy requirements for FR2-NTN	1566

10.1.20C.1	SSB based L1-RSRP accuracy requirements	1566

10.1.20C.1.1	Absolute Accuracy	1566

10.1.20C.1.2	Relative Accuracy	1566

10.1.20C.2	CSI-RS based L1-RSRP accuracy requirements	1567

10.1.20C.2.1	Absolute Accuracy	1567

10.1.20C.2.2	Relative Accuracy	1568

10.1.21	SFTD accuracy requirements	1569

10.1.21.1	SFTD acuracy requirements for NE-DC	1569

10.1.21.2	SFTD acuracy requirements for NR-DC	1570

10.1.21.3	Inter-frequency SFTD acuracy requirements	1571

10.1.22	CLI measurement accuracy requirements	1573

10.1.22.1	SRS-RSRP	1573

10.1.22.1.1	SRS-RSRP Accuracy	1573

10.1.22.1.2	SRS-RSRP report mapping	1574

10.1.22.2	CLI-RSSI	1575

10.1.22.2.1	CLI-RSSI Accuracy	1575

10.1.22.2.2	CLI-RSSI report mapping	1576

10.1.23	RSTD Measurements	1576

10.1.23.1	Introduction	1576

10.1.23.2	Measurement Accuracy Requirements	1576

10.1.23.3	Report mapping	1584

10.1.23.3.1	Absolute DL RSTD Measurement Reporting	1584

10.1.23.3.2	Differential Reporting for DL RSTD Measurement	1587

10.1.23.3.3	Additional Path Report Mapping for DL RSTD	1590

10.1.23A	RSTD Measurements Based on PRS Aggregation	1594

10.1.23A.1	Introduction	1594

10.1.23A.3	Report Mapping	1601

10.1.23A.3.1	Absolute DL RSTD Measurement Reporting	1601

10.1.23A.3.2	Differential Reporting for DL RSTD Measurement	1601

10.1.23A.3.3	Additional Path Report Mapping for DL RSTD	1601

10.1.24	PRS-RSRP Measurements	1601

10.1.24.1	Introduction	1601

10.1.24.2	Measurement Accuracy Requirements	1602

10.1.24.2.1	Absolute PRS-RSRP accuracy	1602

10.1.24.2.2	Relative PRS RSRP accuracy	1606

10.1.24.3	Report mapping	1610

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	1610

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	1611

10.1.24A	PRS-RSRP Measurements Based on PRS Aggregation	1613

10.1.24A.1	Introduction	1613

10.1.24A.2	Measurement Accuracy Requirements	1614

10.1.24A.2.1	Absolute PRS RSRP Accuracy Requirement	1614

10.1.24A.2.2	Relative PRS RSRP Accuracy Requirement	1614

10.1.24A.3	Report Mapping	1614

10.1.24A.3.1	Absolute PRS-RSRP Measurement Report Mapping	1614

10.1.24A.3.2	Differential Report Mapping for PRS-RSRP Measurement	1614

10.1.25	UE Rx-Tx Time Difference Measurements	1614

10.1.25.1	Introduction	1614

10.1.25.2	Measurement Accuracy Requirements	1614

10.1.25.3	Report mapping	1626

10.1.25.3.1	Absolute UE Rx-Tx Measurement Report Mapping	1626

10.1.25.3.2	Differential UE Rx-Tx Measurement Report Mapping	1629

10.1.25.3.3	Additional Path Report Mapping for UE Rx-Tx Time Difference	1632

10.1.25A	UE Rx-Tx Time Difference Measurement Based on PRS Aggregation	1635

10.1.25A.1	Introduction	1635

10.1.25A.2	Measurement Accuracy Requirements	1636

10.1.25A.3	Report mapping	1652

10.1.25C	UE Rx-Tx Time Difference Measurements in Satellite Accesss	1652

10.1.25C.1	Introduction	1652

10.1.25C.2	Measurement Accuracy Requirements	1652

10.1.25C.3	Report mapping	1653

10.1.26	FR2 P-MPR report	1653

10.1.26.1	Report mapping	1653

10.1.27	L1-SINR accuracy requirements for FR1	1654

10.1.27.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured	1654

10.1.27.1.1	Absolute Accuracy	1654

10.1.27.1.2	Relative Accuracy	1655

10.1.27.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured	1656

10.1.27.2.1	Absolute Accuracy	1656

10.1.27.2.2	Relative Accuracy	1658

10.1.27.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated IMR configured	1660

10.1.27.3.1	Absolute Accuracy	1660

10.1.27.3.2	Relative Accuracy	1662

10.1.28	L1-SINR accuracy requirements for FR2	1664

10.1.29	Intra-frequency RSRQ accuracy requirements under CCA	1671

10.1.29.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	1671

10.1.29.1.1	Absolute SS-RSRQ Accuracy	1671

10.1.30	Inter-frequency RSRQ accuracy requirements under CCA	1671

10.1.30.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	1671

10.1.30.1.1	Absolute SS-RSRQ Accuracy	1671

10.1.30.1.2	Relative SS-RSRQ Accuracy	1672

10.1.31	Intra-frequency SINR accuracy requirements under CCA	1673

10.1.31.1	Intra-frequency SS-SINR accuracy requirements in FR1	1673

10.1.31.1.1	Absolute SS-SINR Accuracy	1673

10.1.32	Inter-frequency SINR accuracy requirements under CCA	1673

10.1.32.1	Inter-frequency SS-SINR accuracy requirements in FR1	1673

10.1.32.1.1	Absolute SS-SINR Accuracy	1673

10.1.32.1.2	Relative SS-SINR Accuracy	1674

10.1.33	L1-RSRP accuracy requirements under CCA	1675

10.1.33.1	SSB based L1-RSRP accuracy requirements in FR1	1675

10.1.33.1.1	Absolute Accuracy	1675

10.1.33.1.2	Relative Accuracy	1675

10.1.34	RSSI measurements under CCA	1676

10.1.34.1	Intra-frequency absolute RSSI measurement accuracy requirements in FR1	1676

10.1.34.2	Inter-frequency absolute RSSI measurement accuracy requirements in FR1	1676

10.1.34.3	RSSI measurement report mapping	1676

10.1.35	Channel occupancy measurements under CCA	1677

10.1.35.1	Intra-frequency channel occupancy measurement accuracy requirements in FR1	1677

10.1.35.2	Inter-frequency channel occupancy measurement accuracy requirements in FR1	1677

10.1.36	Intra-frequency RSRP accuracy requirements under CCA	1677

10.1.36.1	Intra-frequency SS-RSRP accuracy requirements in FR1	1677

10.1.36.1.1	Absolute SS-RSRP Accuracy	1677

10.1.36.1.2	Relative SS-RSRP Accuracy	1678

10.1.37	Inter-frequency RSRP accuracy requirements under CCA	1678

10.1.37.1	Inter-frequency SS-RSRP accuracy requirements in FR1	1678

10.1.37.1.1	Absolute SS-RSRP	1678

10.1.37.1.2	Relative SS-RSRP Accuracy	1679

10.1.38	PRS-RSRPP Measurements	1680

10.1.38.1	Introduction	1680

10.1.38.2	Measurement Accuracy Requirements	1680

10.1.38.2.1	Absolute PRS RSRPP accuracy	1680

10.1.38.3	Report mapping	1684

10.1.38.3.1	Absolute PRS-RSRPP Measurement Report Mapping	1684

10.1.38.3.2	Differential Report Mapping for PRS-RSRPP Measurement	1685

10.1.38A	PRS-RSRPP Measurements Based on PRS Aggregation	1686

10.1. 38A.1	Introduction	1686

10.1.38A.2	Measurement Accuracy Requirements	1687

10.1.38A.2.1	Absolute PRS RSRPP accuracy	1687

10.1.38A.3	Report mapping	1687

10.1.38A.3.1	Absolute PRS-RSRPP Measurement Report Mapping	1687

10.1.38A.3.2	Differential Report Mapping for PRS-RSRPP Measurement	1687

10.1.39	UE Rx-Tx time difference measurements for RTT-based PDC	1687

10.1.39.1	Void	1687

10.1.39.2	 Measurement Accuracy Requirements for PRS	1687

10.1.39.3	 Measurement Accuracy Requirements for TRS	1690

10.1.40	Void	1694

10.1.41	FR1 DPC report	1694

10.1.41.1	Report mapping	1694

10.1.42	TDCP Measurement Report Mapping	1694

10.1.43	DL-RSCPD Measurements	1696

10.1.43.1	Introduction	1696

10.1.43.2.1	Measurement Accuracy Requirements	1696

10.1.43.3	Report Mapping	1703

10.1.43.3.1	Absolute DL RSCPD Measurement Reporting	1703

10.1.44	DL-RSCP Measurements	1704

10.1.44.1	Introduction	1704

10.1.44.2	Measurement Accuracy Requirements	1704

10.1.44.3	Report Mapping	1712

10.1.44.3.1	Relative DL RSCP Measurement Reporting	1712

10.1A	NR measurements for RedCap	1713

10.1A.1	Introduction	1713

10.1A.2	Intra-frequency RSRP accuracy requirements for FR1	1713

10.1A.2.1	Intra-frequency SS-RSRP accuracy requirements	1713

10.1A.2.1.1	Absolute SS-RSRP Accuracy	1713

10.1A.2.1.2	Relative SS-RSRP Accuracy	1714

10.1A.3	Intra-frequency RSRP accuracy requirements for FR2	1715

10.1A.3.1	Intra-frequency SS-RSRP accuracy requirements	1715

10.1A.3.1.1	Absolute SS-RSRP Accuracy	1715

10.1A.3.1.2	Relative SS-RSRP Accuracy	1715

10.1A.4	Inter-frequency RSRP accuracy requirements for FR1	1715

10.1A.4.1	Inter-frequency SS-RSRP accuracy requirements	1715

10.1A.4.1.1	Absolute SS-RSRP Accuracy in FR1	1715

10.1A.4.1.2	Relative SS-RSRP Accuracy in FR1	1716

10.1A.5	Inter-frequency RSRP accuracy requirements for FR2	1717

10.1A.5.1	Inter-frequency SS-RSRP accuracy requirements	1717

10.1A.5.1.1	Absolute SS-RSRP Accuracy	1717

10.1A.5.1.2	Relative SS-RSRP Accuracy	1717

10.1A.6	Intra-frequency RSRQ accuracy requirements for FR1	1717

10.1A.6.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	1717

10.1A.6.1.1	Absolute SS-RSRQ Accuracy in FR1	1717

10.1A.7	Intra-frequency RSRQ accuracy requirements for FR2	1718

10.1A.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	1718

10.1A.7.1.1	Absolute SS-RSRQ Accuracy in FR2	1718

10.1A.8	Inter-frequency RSRQ accuracy requirements for FR1	1718

10.1A.8.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	1718

10.1A.8.1.1	Absolute SS-RSRQ in FR1	1718

10.1A.8.1.2	Relative SS-RSRQ Accuracy in FR1	1719

10.1A.9	Inter-frequency RSRQ accuracy requirements for FR2	1720

10.1A.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	1720

10.1A.9.1.1	Absolute SS-RSRQ Accuracy in FR2	1720

10.1A.9.1.2	Relative SS-RSRQ Accuracy in FR2	1720

10.1A.10	 Intra-frequency SINR accuracy requirements for FR1	1720

10.1A.10.1	Intra-frequency SS-SINR accuracy requirements in FR1	1720

10.1A.10.1.1	Absolute SS-SINR Accuracy in FR1	1720

10.1A.11	Intra-frequency SINR accuracy requirements for FR2	1721

10.1A.11.1	Intra-frequency SS-SINR accuracy requirements in FR2	1721

10.1A.11.1.1	Absolute SS-SINR Accuracy in FR2	1721

10.1A.12 	Inter-frequency SINR accuracy requirements for FR1	1721

10.1A.12.1	Inter-frequency SS-SINR accuracy requirements in FR1	1721

10.1A.12.1.1	Absolute SS-SINR Accuracy in FR1	1721

10.1A.12.1.2	Relative SS-SINR Accuracy in FR1	1722

10.1A.13	 Inter-frequency SINR accuracy requirements for FR2	1723

10.1A.13.1	Inter-frequency SS-SINR accuracy requirements in FR2	1723

10.1A.13.1.1	Absolute SS-SINR Accuracy in FR2	1723

10.1A.13.1.2	Relative SS-SINR Accuracy in FR2	1723

10.1A.14	L1-RSRP accuracy requirements for FR1	1723

10.1A.14.1	SSB based L1-RSRP accuracy requirements	1723

10.1A.14.1.1	Absolute Accuracy	1723

10.1A.14.1.2	Relative Accuracy	1724

10.1A.14.2	CSI-RS based L1-RSRP accuracy requirements	1725

10.1A.14.2.1	Absolute Accuracy	1725

10.1A.14.2.2	Relative Accuracy	1726

10.1A.15	 L1-RSRP accuracy requirements for FR2	1727

10.1A.15.1	SSB based L1-RSRP accuracy requirements	1727

10.1A.15.1.1	Absolute Accuracy	1727

10.1A.15.1.2	Relative Accuracy	1727

10.1A.15.2	CSI-RS based L1-RSRP accuracy requirements	1727

10.1A.15.2.1	Absolute Accuracy	1727

10.1A.15.2.2	Relative Accuracy	1727

10.1A.16	RSTD Measurements for RedCap Positioning	1728

10.1A.16.1	Introduction	1728

10.1A.16.2	Measurement Accuracy Requirements	1728

10.1A.16.2.1	Accuracy requirement for RSTD measurement without RX FH	1728

10.1A.16.2.2	Accuracy requirement for RSTD measurement with RX FH	1735

10.1A.16.3	Report Mapping	1750

10.1A.16.3.1	Absolute DL RSTD Measurement Reporting	1750

10.1A.16.3.2	Differential Reporting for DL RSTD Measurement	1750

10.1A.16.3.3	Additional Path Report Mapping for DL RSTD	1750

10.1A.17	PRS-RSRP Measurements for RedCap positioning	1750

10.1A.17.1	Introduction	1750

10.1A.17.2	Measurement Accuracy Requirements	1750

10.1A.17.2.1	Absolute PRS RSRP Accuracy Requirement	1750

10.1A.17.2.2	Relative PRS RSRP Accuracy Requirement	1753

10.1A.17.3	Report Mapping	1753

10.1A.17.3.1	Absolute PRS-RSRP Measurement Report Mapping	1753

10.1A.17.3.2	Differential Report Mapping for PRS-RSRP Measurement	1753

10.1A.18	  UE Rx-Tx Time Difference Measurements for RedCap Positioning	1754

10.1A.18.1	Introduction	1754

10.1A.18.2	Measurement Accuracy Requirements	1754

10.1A.18.2.1	UE Rx-Tx Accuracy Requirement for 2RX RedCap UE without FH	1754

10.1A.18.2.2	UE Rx-Tx Accuracy Requirement for 1RX RedCap UE without FH	1755

10.1A.18.2.3	UE Rx-Tx Accuracy Requirement for 2RX RedCap UE with FH	1761

10.1A.18.3	Report mapping	1771

10.1A.18.3.1	Absolute UE Rx-Tx Measurement Report Mapping	1771

10.1A.18.3.2	Differential UE Rx-Tx Measurement Report Mapping	1771

10.1A.18.3.3	Additional Path Report Mapping for UE Rx-Tx Time Difference	1771

10.1A.19	PRS-RSRPP Measurements for RedCap Positioning	1771

10.1A.19.1	Introduction	1771

10.1A.19.2	Measurement Accuracy Requirements	1772

10.1A.19.2.1	Absolute PRS RSRPP accuracy	1772

10.1A.19.3	Report mapping	1774

10.1A.19.3.1	Absolute PRS-RSRPP Measurement Report Mapping	1774

10.1A.19.3.2	Differential Report Mapping for PRS-RSRPP Measurement	1775

10.2	E-UTRAN measurements	1775

10.2.1	Introduction	1775

10.2.2	E-UTRAN RSRP measurements	1775

10.2.3	E-UTRAN RSRQ measurements	1775

10.2.4	E-UTRAN RSTD measurements	1775

10.2.5	E-UTRAN RS-SINR measurements	1776

10.2.6	E-UTRAN RSRP measurements for CA/DC Idle Mode Measurements	1776

10.2.7	E-UTRAN RSRQ measurements for CA/DC Idle Mode Measurements	1776

10.2A	E-UTRAN measurements for RedCap	1777

10.2A.1	Introduction	1777

10.2A.2	E-UTRAN RSRP measurements	1777

10.2A.3	E-UTRAN RSRQ measurements	1777

10.2A.4	E-UTRAN RS-SINR measurements	1778

10.3	UTRAN FDD Measurements	1778

10.3.1	UTRAN FDD CPICH RSCP	1778

10.3.2	UTRAN FDD CPICH Ec/No	1779

10.4	V2X measurements	1779

10.4.1	Introduction	1779

10.4.2	Intra-frequency PSBCH-RSRP accuracy requirements for FR1	1779

10.4.2.1	PSBCH-RSRP Absolute Accuracy	1779

10.4.2.2	PSBCH-RSRP Relative Accuracy	1780

10.4.2A	Intra-frequency PSBCH-RSRP accuracy requirements for FR1 under CCA	1781

10.4.2A.1	PSBCH-RSRP Absolute Accuracy	1781

10.4.2A.2	PSBCH-RSRP Relative Accuracy	1781

10.4.3	Intra-Frequency SL-RSSI Measurement Accuracy Requirements for FR1	1782

10.4.3.1	Absolute SL-RSSI Accuracy	1782

10.4.3A	Intra-Frequency SL-RSSI Measurement Accuracy Requirements for FR1 under CCA	1782

10.4.3A.1	Absolute SL-RSSI Accuracy	1782

10.4.4	Intra-Frequency L1 SL-RSRP Measurement Accuracy Requirements for FR1	1783

10.4.4.1	Absolute L1 SL-RSRP Accuracy	1783

10.4.4A	Intra-Frequency L1 SL-RSRP Measurement Accuracy Requirements for FR1 under CCA	1783

10.4.4A.1	Absolute L1 SL-RSRP Accuracy	1783

10.4.5	Intra-Frequency Discovery Signal Measurement Accuracy Requirements	1784

10.4.5.1	Absolute Discovery Signal Measurement Accuracy	1784

10.4A	NR Sidelink Measurements for Positioning	1785

10.4A.1	Introduction	1785

10.4A.2	SL RSTD measurements	1785

10.4A.2.1	Measurement Report Mapping	1785

10.4A.2.1.1	Absolute SL RSTD Measurement Reporting	1785

10.4A.2.2	Measurement Accuracy Requirements	1786

10.4A.3	SL PRS-RSRP measurements	1788

10.4A.3.1	Measurement Report Mapping	1788

10.4A.3.1.1	Absolute SL PRS-RSRP Measurement Report Mapping	1788

10.4A.3.2	Measurement Accuracy Requirements	1789

10.4A.3.2.1	Absolute SL PRS-RSRP accuracy	1789

10.4A.4	SL Rx-Tx measurements	1790

10.4A.4.1	Measurement Report Mapping	1790

10.4A.4.1.1	Absolute SL Rx-Tx Measurement Report Mapping	1790

10.4A.4.2	Measurement Accuracy	1792

10.4A.5	SL PRS-RSRPP measurements	1793

10.4A.5.1	Measurement Report Mapping	1793

10.4A.5.1.1	Absolute SL PRS-RSRPP Measurement Report Mapping	1793

10.4A.5.2	Measurement Accuracy	1794

10.4A.5.2.1	Introduction	1794

10.4A.5.2.2	Measurement Accuracy Requirements	1795

10.4A.5.2.2.2	Absolute SL PRS-RSRPP accuracy	1795

10.4A.6	SL AoA measurements	1796

10.4A.6.1	Measurement Report Mapping	1796

10.4A.6.1.1	Absolute SL AoA Measurement Report Mapping	1796

10.4A.7	SL RTOA measurements	1797

10.4A.7.1	Measurement Report Mapping	1797

10.4A.7.1.1	Absolute SL RTOA Measurement Report Mapping	1797

11	Void	1799

12	V2X Requirements	1800

12.1	Introduction	1800

12.2	UE Transmit Timing	1800

12.2.1	Introduction	1800

12.2.2	GNSS as synchronization reference source	1801

12.2.3	NR Cell as synchronization reference source	1801

12.2.4	E-URTAN Cell as synchronization reference source	1801

12.2.5	SyncRef UE as synchronization reference source	1802

12.3	Initiation/Cease of SLSS Transmissions	1802

12.3.1	Introduction	1802

12.3.1.1	Initiation/Cease of SLSS transmissions with NR cell as synchronization reference source	1802

12.3.1.2	Initiation/Cease of SLSS transmissions with EUTRAN cell as synchronization reference source	1803

12.3.1.3	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	1804

12.3.1.4	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	1804

12.3A	Initiation/Cease of SLSS Transmissions with CCA	1805

12.3A.1	Introduction	1805

12.3A.1.1	Initiation/Cease of SLSS transmissions with NR cell as synchronization reference source	1805

12.3A.1.2	Initiation/Cease of SLSS transmissions with EUTRAN cell as synchronization reference source	1805

12.3A.1.3	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	1805

12.3A.1.4	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	1806

12.4	Selection / Reselection of V2X Synchronization Reference Source	1806

12.4A	Selection / Reselection of Sidelink Synchronization Reference Source with CCA	1808

12.5	L1 SL-RSRP measurements	1810

12.5.1	Introduction	1810

12.5.2	SL-RSRP measurements	1810

12.6	Congestion Control measurements	1811

12.7	Interruption	1811

12.7.1	Interruptions to WAN due to V2X Sidelink Communication	1811

12.7.2	V2X Sidelink Communication Dropping due to synchronization source change	1811

12.7.3	Interruptions to WAN due to switching between E-UTRA V2X Sidelink and NR V2X Sidelink	1813

12.7.4	Interruptions to WAN at transitions between active and non-active during SL-DRX	1813

12.7.5	Interruptions to V2X sidelink at transitions between active and non-active during DRX	1814

12.7.6	Interruptions to V2X sidelink due to Active BWP switching Requirement	1814

12.7.7	Interruptions to WAN due to SyncRef UE detection and/or Sensing during SL DRX off duration	1815

12.7.8	Interruptions at NR sidelink discovery configuration	1815

12.7.9	Interruptions to WAN due to sidelink carrier addition/release	1815

12.8	Reliability of GNSS signal	1816

12.9	Scheduling availability	1816

12.9.1	Scheduling availability of UE switching between E-UTRA sidelink and NR sidelink	1816

12.9.2	Scheduling availability of UE switching between Uu uplink  and V2X sidelink	1816

12.10	Selection / Reselection of relay UE	1817

12.10.1	Introduction	1817

12.10.2	Selection / Reselection of relay UE	1817

12.11	Component Carrier Addition and Release Delay for Sidelink Carrier Aggregation	1817

12.12	Selection / Reselection of Synchronization Reference Source for NR SL Carrier Aggregation	1818

12A	NR Sidelink Measurements for Positioning	1819

12A.1	Introduction	1819

12A.2	SL RSTD measurements	1820

12A.2.1	Introduction	1820

12A.2.3	Measurement Capability	1820

12A.2.4	Measurement Reporting Requirements	1820

12A.2.5	Measurements Period Requirements	1820

12A.3	SL PRS-RSRP measurements	1821

12A.3.1	Introduction	1821

12A.3.2	Requirements Applicability	1822

12A.3.4	Measurement Reporting Requirements	1822

12A.3.5	Measurements Period Requirements	1822

12A.4	SL Rx-Tx measurements	1823

12A.4.1	Introduction	1823

12A.4.2	Requirements Applicability	1823

12A.4.3	Measurement Capability	1823

12A.4.4	Measurement Reporting Requirements	1823

12A.4.5	Measurement Period Requirements	1824

12A.5	SL PRS-RSRPP measurements	1825

12A.5.1	Introduction	1825

12A.5.2	Requirements Applicability	1825

12A.5.3	Measurement Capability	1825

12A.5.4	Measurement Reporting Requirements	1825

12A.5.5	Measurement Period Requirements	1825

12A.6	SL AoA measurements	1826

12A.6.1	Introduction	1826

12A.6.2	Requirements Applicability	1826

12A.6.3	Measurement Capability	1826

12A.6.4	Measurement Reporting Requirements	1826

12A.6.5	Measurement Period Requirements	1827

12A.7	SL RTOA measurements	1827

12A.7.1	Introduction	1827

12A.7.2	Requirements Applicability	1828

12A.7.3	Measurement Capability	1828

12A.7.4	Measurement Reporting Requirements	1828

12A.7.5	Measurement Period Requirements	1828

13	Measurement Performance Requirements for NR gNB	1829

13.1	UL-RTOA	1829

13.1.1	Report mapping	1829

13.1.1A	Additional Path Report Mapping for UL-RTOA	1833

13.2	gNB Rx-Tx time difference	1836

13.2.1	Report mapping	1836

13.2.1A	Additional Path Report Mapping for gNB Rx-Tx	1840

13.2.2	Measurement Accuracy Requirements	1843

13.2.2.1	Introduction	1843

13.2.2.2	Requirements	1844

13.3	UL SRS RSRP measurement	1845

13.3.1	Report mapping	1845

13.3.2	Measurement accuracy requirements	1845

13.3.2.1	Introduction	1845

13.3.2.2	Requirements	1846

13.4	AoA/ZoA	1846

13.4.1	Report mapping	1846

13.5	Timing advance (TADV)	1847

13.5.1	Report mapping	1847

13.6	UL SRS RSRPP measurement	1848

13.6.1	Report mapping	1848

13.7	gNB Rx-Tx time difference measurements for RTT-based PDC	1848

13.7.1	Report mapping	1848

13.7.2	Measurement Accuracy Requirements	1849

13.7.2.1	Introduction	1849

13.7.2.2	Requirements	1849

13.8	UL-RSCP measurement	1850

13.8.1	Report mapping	1850

13.9	UL SRS-TDCT measurement	1850

13.9.1	Report mapping	1850

13.10	UL SRS-TDCP measurement	1853

13.10.1	Report mapping	1853

Annex A (normative):	Test Cases	1855

A.1	Purpose of annex	1855

A.2	Requirement classification for statistical testing	1855

A.2.1	Types of requirements in TS 38.133	1855

A.2.1.1	Time and delay requirements on UE higher layer actions	1855

A.2.1.2	Measurements of power levels, relative powers and time	1856

A.2.1.3	Implementation requirements	1856

A.2.1.4	Physical layer timing requirements	1856

A.2.1.5	Requirements under CCA	1856

A.3	RRM test configurations	1857

A.3.1	Reference measurement channels	1857

A.3.1.1	PDSCH	1857

A.3.1.1.1	FDD	1857

A.3.1.1.2	TDD	1858

A.3.1.2	CORESET for RMSI scheduling	1861

A.3.1.2.1	FDD	1861

A.3.1.2.2	TDD	1862

A.3.1.3	CORESET for RMC scheduling	1864

A.3.1.3.1	FDD	1864

A.3.1.3.2	TDD	1866

A.3.1.4	TDD UL/DL configuration	1870

A.3.1A	Reference measurement channels under CCA	1872

A.3.1A.1	PDSCH	1872

A.3.1A.1.1	TDD	1872

A.3.1A.2	CORESET for RMSI scheduling	1873

A.3.1A.2.1	TDD	1873

A.3.1A.3	CORESET for RMC scheduling	1874

A.3.1A.3.1	TDD	1874

A.3.1A.4	TDD UL/DL configuration	1874

A.3.1A.5	RMC burst transmission model	1875

A.3.2.1	Generic OFDMA Channel Noise Generator (OCNG)	1875

A.3.2.1.1	OCNG pattern 1: Generic OCNG pattern for all unused REs	1875

A.3.2.1.2	OCNG pattern 2: Generic OCNG pattern for all unused REs for 2AoA setup	1876

A.3.2.1.3	OCNG pattern 3: Generic OCNG pattern for unused REs in the same bandwidth as CORESET	1876

A.3.2.1.4	OCNG pattern 4: Generic OCNG pattern for all unused REs outside SSB slot(s)	1877

A.3.2.2	Void	1878

A.3.3	Reference DRX configurations	1878

A.3.3.1	DRX Configuration 1: DRX cycle = 40 ms and TAT = 500 ms	1878

A.3.3.2	DRX Configuration 2: DRX cycle = 640 ms and TAT = 500 ms	1878

A.3.3.3	DRX Configuration 3: DRX cycle = 40 ms and TAT = Infinity	1878

A.3.3.4	DRX Configuration 4: DRX cycle = 160 ms and TAT = Infinity	1879

A.3.3.5	DRX Configuration 5: DRX cycle = 320 ms and TAT = Infinity	1879

A.3.3.6	DRX Configuration 6: DRX cycle = 320 ms and TAT = 500 ms	1879

A.3.3.7	DRX Configuration 7: DRX cycle = 640 ms and TAT = Infinity	1879

A.3.3.8	DRX Configuration 8: DRX cycle = 320 ms and TAT = Infinity	1880

A.3.3.9	DRX Configuration 9: DRX cycle = 40 ms and TAT = 500 ms	1880

A.3.3.10	DRX Configuration 10: DRX cycle = 640 ms and TAT = 500 ms	1880

A.3.3.11	DRX Configuration 11: DRX cycle = 20 ms and TAT = Infinity	1880

A.3.3.12	DRX Configuration 12: DRX cycle = 640 ms and TAT = Infinity	1881

A.3.3.13	DRX Configuration X1: DRX cycle = 80 ms and TAT = Infinity	1881

A.3.3.14	DRX Configuration 14: DRX cycle = 160 ms and TAT = Infinity	1881

A.3.4	Test Cases with Different Channel Bandwidths	1881

A.3.4.1	Test Cases with Different E-UTRA Channel Bandwidths	1881

A.3.4.1.1	Introduction	1881

A.3.4.1.2	Principle of testing	1882

A.3.5	Test Cases for Synchronous and Asynchronous DC Operations	1882

A.3.5.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations	1882

A.3.5.1.1	Introduction	1882

A.3.5.1.2	Principle of Testing	1882

A.3.6	Antenna configurations	1882

A.3.6.1	Antenna configurations for FR1	1882

A.3.6.1.1	Antenna connection for 4 Rx capable UEs	1882

A.3.6.1.1.1	Introduction	1882

A.3.6.1.1.2	Principle of testing	1882

A.3.6.1.2	Antenna connection for 8 Rx capable UEs	1885

A.3.6.1.2.1	Introduction	1885

A.3.6.1.2.2	Principle of testing	1885

A.3.6.1.3	Antenna connection for 6 Rx capable UEs	1887

A.3.6.1.3.1	Introduction	1887

A.3.6.1.3.2	Principle of testing	1887

A.3.6.2	Antenna configurations for FR2	1888

A.3.6A	Antenna configurations with unlicensed bands	1888

A.3.6A.1	Antenna configurations for FR1	1888

A.3.6A.1.1	Antenna connection for 4 Rx capable UEs	1888

A.3.6A.1.1.1	Introduction	1888

A.3.6A.1.1.2	Principle of testing	1888

A.3.7	EN-DC test setup	1890

A.3.7.1	Introduction	1890

A.3.7.2	E-UTRAN Serving Cell Parameters	1890

A.3.7.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	1890

A.3.7.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	1891

A.3.7A	NR FR1-FR2 test setup	1892

A.3.7B	EN-DC test setup with unlicensed bands	1892

A.3.7B.1	Introduction	1892

A.3.7B.2	E-UTRAN Serving Cell Parameters	1892

A.3.7B.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) under CCA in FR1	1892

A.3.7C	LTE-FR1/FR2 test setup	1893

A.3.7D	NE-DC test setup	1894

A.3.7D.1	Introduction	1894

A.3.7D.2	E-UTRAN Serving Cell Parameters	1894

A.3.7D.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	1894

A.3.7D.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	1894

A.3.8	PRACH configurations	1894

A.3.8.1	Introduction	1894

A.3.8.2	PRACH configurations in FR1	1894

A.3.8.2.1	FR1 PRACH configuration 1	1894

A.3.8.2.2	FR1 PRACH configuration 2	1895

A.3.8.2.3	FR1 PRACH configuration 3	1895

A.3.8.2.4	FR1 PRACH configuration 4	1896

A.3.8.2.5	FR1 PRACH configuration 5	1896

A.3.8.2.6	FR1 PRACH configuration 6	1897

A.3.8.3	PRACH configurations in FR2	1897

A.3.8.3.1	FR2 PRACH configuration 1	1897

A.3.8.3.2	FR2 PRACH configuration 2	1898

A.3.8.3.3	FR2 PRACH configuration 3	1899

A.3.8.3.4	FR2 PRACH configuration 4	1899

A.3.8.3.5	FR2 PRACH configuration 5	1900

A.3.8.3.6	FR2 PRACH configuration 6	1900

A.3.8A	PRACH configurations under CCA	1901

A.3.8A.1	Introduction	1901

A.3.8A.2	PRACH configurations in FR1	1901

A.3.8A.2.1	FR1 PRACH configuration 1 under CCA	1901

A.3.8A.2.2	FR1 PRACH configuration 2 under CCA	1902

A.3.9	BWP configurations	1903

A.3.9.1	Introduction	1903

A.3.9.2	Downlink BWP configurations	1903

A.3.9.2.1	Initial BWP	1903

A.3.9.2.2	Dedicated BWP	1904

A.3.9.3	Uplink BWP configurations	1904

A.3.9.3.1	Initial BWP	1904

A.3.9.3.2	Dedicated BWP	1905

A.3.9A	BWP configurations for RedCap	1905

A.3.9A.1	Introduction	1905

A.3.9A.2	Downlink BWP configurations	1905

A.3.9A.2.1	Dedicated BWP	1905

A.3.9A.3	Uplink BWP configurations	1906

A.3.9A.3.1	Dedicated BWP	1906

A.3.10	SSB Configurations	1906

A.3.10.1	SSB Configurations for FR1	1906

A.3.10.1.1	SSB pattern 1 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1906

A.3.10.1.5	SSB pattern 5 in FR1: SSB allocation for SSB SCS=15 kHz starting from odd SFN in 10 MHz	1908

A.3.10.1.6	SSB pattern 6 in FR1: SSB allocation for SSB SCS=30 kHz starting from odd SFN in 40 MHz	1908

A.3.10.1.7	SSB pattern 7 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1908

A.3.10.1.8	SSB pattern 8 in FR1: SSB allocation for SSB SCS=30 kHz in 40 MHz	1909

A.3.10.1.9	SSB pattern 9 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1909

A.3.10.1.10	SSB pattern 10 in FR1: SSB allocation for SSB SCS=30 kHz in 40 MHz	1909

A.3.10.1.11	SSB pattern 11 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1910

A.3.10.1.12	SSB pattern 12 in FR1: SSB allocation for SSB SCS=30 kHz in 40 MHz	1910

A.3.10.1.13	SSB pattern 13 in FR1: SSB allocation for SSB SCS=15 kHz in 3 MHz	1910

A.3.10.2	SSB Configurations for FR2	1911

A.3.10.2.1	SSB pattern 1 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1911

A.3.10.2.2	SSB pattern 2 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1911

A.3.10.2.3	SSB pattern 3 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1912

A.3.10.2.4	SSB pattern 4 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1912

A.3.10.2.5	SSB pattern 5 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1912

A.3.10.2.6	SSB pattern 6 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1913

A.3.10.2.7	SSB pattern 7 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1913

A.3.10.2.8	SSB pattern 8 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1913

A.3.10.2.9	SSB pattern 9 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1914

A.3.10.2.10	SSB pattern 10 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1914

A.3.10.2.19	SSB pattern 19 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1918

A.3.10.2.20	SSB pattern 20 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1919

A.3.10.2.21	SSB pattern 21 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1919

A.3.10.2.22	SSB pattern 22 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1920

A.3.10.2.23	SSB pattern 23 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1920

A.3.10.2.24	SSB pattern 24 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1920

A.3.10.2.25	SSB pattern 25 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1921

A.3.10.2.26	SSB pattern 26 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1921

A.3.10.2.27	SSB pattern 27 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1922

A.3.10A	SSB Configurations under CCA	1922

A.3.10A.1	SSB Configurations under CCA for FR1	1922

A.3.10A.1.1	SSB pattern 1 under CCA for semi-static channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	1922

A.3.10A.1.2	SSB pattern 2 under CCA for dynamic channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	1923

A.3.10A.1.3	SSB pattern 3 under CCA for semi-static channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	1923

A.3.10A.1.4	SSB pattern 4 under CCA for dynamic channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	1924

A.3.10B	SSB Configurations for RedCap	1924

A.3.10B.1	SSB Configurations for FR1	1924

A.3.10B.1.1	SSB pattern 1 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1924

A.3.10B.1.2	SSB pattern 2 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1925

A.3.10B.1.3	SSB pattern 3 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz starting from odd SFN in 20 MHz	1925

A.3.10B.1.4	SSB pattern 4 for RedCap in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1926

A.3.10B.1.5	SSB pattern 5 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1926

A.3.10B.1.6	SSB pattern 6 for RedCap in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1927

A.3.10B.1.7	SSB pattern 7 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1927

A.3.10B.2	SSB Configurations for FR2	1928

A.3.10B.2.1	SSB pattern 1 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1928

A.3.10B.2.2	SSB pattern 2 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1928

A.3.10B.2.3	SSB pattern 3 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1929

A.3.10B.2.4	SSB pattern 4 for RedCap in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1929

A.3.10B.2.5	SSB pattern 5 for RedCap in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1930

A.3.11	SMTC Configurations	1930

A.3.11.1	SMTC pattern 1: SMTC period = 20 ms with SMTC duration = 1 ms	1930

A.3.11.2	SMTC pattern 2: SMTC period = 20 ms with SMTC duration = 5 ms	1930

A.3.11.3	SMTC pattern 3: SMTC period = 160 ms with SMTC duration = 1 ms	1930

A.3.11.4	SMTC pattern 4: SMTC period = 20 ms with SMTC duration = 1 ms	1931

A.3.11.5	SMTC pattern 5: SMTC period = 20 ms with SMTC duration = 5 ms	1931

A.3.11.6	SMTC pattern 6: SMTC period = 20 ms with SMTC duration = 5 ms	1931

A.3.11.7	SMTC pattern 7: SMTC period = 20 ms with SMTC duration = 5 ms	1931

A.3.11.8	SMTC pattern 8: SMTC period = 10 ms with SMTC duration = 1 ms	1931

A.3.11.9	SMTC pattern 9: SMTC period = 20 ms with SMTC duration = 1 ms	1931

A.3.11.10	SMTC pattern 10: SMTC period = 80 ms with SMTC duration = 1 ms	1932

A.3.11.11	SMTC pattern 11: SMTC period = 80 ms with SMTC duration = 5 ms	1932

A.3.11.12	SMTC pattern 12: SMTC period = 20 ms with SMTC duration = 5 ms	1932

A.3.11A	SMTC Configurations for RedCap	1932

A.3.11A.0	Introduction	1932

A.3.11A.1	SMTC pattern 1 for RedCap: SMTC period = 40 ms with SMTC duration = 1 ms	1933

A.3.11A.2	SMTC pattern 2 for RedCap: SMTC period = 80 ms with SMTC duration = 1 ms	1933

A.3.11A.3	SMTC pattern 3 for RedCap: SMTC period = 40 ms with SMTC duration = 1 ms	1933

A.3.11A.4	SMTC pattern 4 for RedCap: SMTC period = 80 ms with SMTC duration = 5 ms	1933

A.3.12	Test Cases with Different CC Configurations	1933

A.3.12.1 EN-DC Test Cases with Different EN-DC Configurations	1933

A.3.12.1.1	Introduction	1933

A.3.12.1.2	Principle of testing	1933

A.3.12.2	Carrier Aggregation Test Cases with Different CA Configurations	1934

A.3.12.2.1	Introduction	1934

A.3.12.2.2	Principle of testing	1934

A.3.13	Test Cases in SA and EN-DC Operations	1934

A.3.13.1	Introduction	1934

A.3.13.2	Principle of Testing	1934

A.3.13B	 Test Cases for EN-DC and NE-DC Operations	1935

A.3.13B.1	Active BWP switch Test Cases for EN-DC and NE-DC Operations	1935

A.3.13B.1.1	Introduction	1935

A.3.13B.1.2	Principle of Testing	1935

A.3.13B.2	SFTD accuracy Test Cases for EN-DC and NE-DC Operations	1935

A.3.13B.2.1	Introduction	1935

A.3.13B.2.2	Principle of Testing	1936

A.3.14	CSI-RS configurations	1936

A.3.14.1	FDD	1936

A.3.14.2	TDD	1938

A.3.15	Angle of Arrival (AoA) for FR2 RRM test cases	1943

A.3.15.1	Setup 1: Single AoA in Rx beam peak direction	1943

A.3.15.2	Setup 2: Single AoA in non Rx beam peak direction	1943

A.3.15.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction	1943

A.3.15.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction	1944

A.3.15.3	Setup 3: 2 AoAs	1944

A.3.15.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak	1944

A.3.15.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction	1944

A.3.15.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction	1944

A.3.15.4.3	Setup 4c: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak for power class 6 UE supporting simultaneous reception from multiple directions	1944

A.3.15.5	Setup 5: 2 AoAs for simultaneous reception with QCL Type-D	1945

A.3.15.6	Setup 6: 3 AoAs for simultaneous reception with different QCL Type-D	1945

A.3.15.7	Setup 7: 3 AoAs	1945

A.3.15C	Angle of Arrival (AoA) for FR2-NTN RRM test cases	1945

A.3.15C.1	Setup 1: Single AoA	1945

A.3.15C.2	Setup 2: 2 AoAs	1946

A.3.16	TCI State Configuration	1946

A.3.16.1	Introduction	1946

A.3.16.2	TCI states	1946

A.3.16A	Unified TCI State Configuration	1946

A.3.16A.1	Introduction	1946

A.3.16A.2	DLorJoint TCI states	1947

A.3.16A.3	UL TCI states	1948

A.3.16B	 LTM Candidate TCI State Configuration	1948

A.3.16B.1	Introduction	1948

A.3.16B.2	LTM candidate DLorJoint TCI states	1949

A.3.16B.3	LTM candidate UL TCI states	1949

A.3.17	Configurations of CSI-RS for tracking	1950

A.3.17.1	Configuration of CSI-RS for tracking for FR1	1950

A.3.17.1.1	FDD	1950

A.3.17.1.2	TDD	1953

A.3.17.2	Configuration of CSI-RS for tracking for FR2	1957

A.3.17.2.1	TDD	1957

A.3.17.2.2	FDD	1960

A.3.18	Additional definitions related to OTA testing for FR2 RRM test cases	1960

A.3.18.1	Introduction	1960

A.3.18.2	PRACH Power Measurement	1961

A.3.19	Test applicability for DAPS handover	1961

A.3.19.1	Introduction	1961

A.3.19.2	Principle of testing	1961

A.3.20	MsgA configurations	1961

A.3.20.1	Introduction	1961

A.3.20.2	MsgA configurations in FR1	1961

A.3.20.2.1	FR1 MsgA configuration 1	1961

A.3.20.2.2	FR1 MsgA configuration 2	1962

A.3.20.3	MsgA configurations in FR2	1963

A.3.20.3.1	FR2 MsgA configuration 1	1963

A.3.20.3.2	FR2 MsgA configuration 2	1964

A.3.20A	MsgA configurations under CCA	1965

A.3.20A.1	Introduction	1965

A.3.20A.2	MsgA configurations in FR1	1965

A.3.20A.2.1	FR1 MsgA configuration 1 under CCA	1965

A.3.20A.2.2	FR1 MsgA configuration 2 under CCA	1966

A.3.21	V2X sidelink communication	1967

A.3.21.1	Introduction	1967

A.3.21.2	Reference resource pool configurations for V2X Sidelink Communication	1968

A.3.21.3	Reference measurement channels for V2X Sidelink Communication	1971

A.3.21.4	Reference SL-DRX configurations	1972

A.3.21.4.1	SL-DRX Configuration 1: SL-DRX cycle = 40 ms	1972

A.3.21.4.2	SL-DRX Configuration 2: SL-DRX cycle = 320 ms	1972

A.3.21.4.3	SL-DRX Configuration 3: SL-DRX cycle = 640 ms	1972

A.3.21A	NR Sidelink Measurements for Positioning	1972

A.3.21A.1	Introduction	1972

A.3.21A.2	NR SL-PRS configurations	1973

A.3.21A.2.1	NR SL-PRS configurations for FR1	1973

A.3.22	CSI-IM configurations	1973

A.3.22.1	FDD	1973

A.3.22.2	TDD	1973

A.3.23	Spatial Relation Configuration	1974

A.3.23.1	Introduction	1974

A.3.23.2	Spatial Relation	1975

A.3.24	SRS configuration	1975

A.3.25	Channel bandwidth (CBW) configurations	1977

A.3.25.1	DL UE specific CBW	1977

A.3.25.2	UL UE specific CBW	1978

A.3.26	CCA model	1978

A.3.26.1	Introduction	1978

A.3.26.2	CCA model for operation on a carrier frequency with CCA in FR1	1978

A.3.26.2.1	DL CCA model	1978

A.3.26.2.2	UL CCA model	1979

A.3.26.3	CCA model for operation on a carrier frequency with CCA in FR2-2	1980

A.3.26.3.1	DL CCA model	1980

A.3.26.3.2	UL CCA model	1980

A.3.26.4	CCA model for operation on a sidelink carrier frequency with CCA	1981

A.3.26.4.1	CCA model for SyncRef UE	1981

A.3.27	Void	1982

A.3.27.1	Void	1982

A.3.27.2	Void	1982

A.3.27.3	Void	1982

A.3.27.4	Void	1982

A.3.27.5	Void	1982

A.3.28	Discovery Burst Transmission Window configuration under CCA	1982

A.3.28.1	DBT Window pattern 1: DBT Window period = 20 ms with DBT Window duration = 1 ms	1982

A.3.29	Testing principles for UE capable of only NR bands with shared spectrum access	1982

A.3.29.1	Introduction	1982

A.3.29.2	Principle of testing for UE capable of EN-DC with only NR bands with shared spectrum access	1982

A.3.29.3	Principle of testing for UE capable of SA operation with only NR bands with shared spectrum access	1983

A.3.30	CSI-RS configurations for RRM	1983

A.3.30.1	FDD	1983

A.3.30.2	TDD	1984

A.3.31	PRS Configurations	1985

A.3.31.1	PRS Configurations for FR1	1985

A.3.31.1.1	PRS pattern 1 in FR1: SCS=15 kHz	1985

A.3.31.1.2	PRS pattern 2 in FR1: SCS=30 kHz	1986

A.3.31.2	PRS Configurations for FR2	1987

A.3.31.2.1	PRS pattern 1 in FR2: SCS=120 kHz	1987

A.3.32	NR sidelink discovery	1987

A.3.32.1	Introduction	1987

A.3.32.2	Reference resource pool configurations for NR Sidelink Discovery	1987

A.3.32.3	Principle of Testing	1988

A.3.33	PRS Processing Window (PPW) configurations	1988

A.3.34	Testing principles for test cases related to PRS measurements	1988

A.3.34.1	Introduction	1988

A.3.34.2	Test cases in RRC_INACTIVE state	1988

A.3.34.3	Test cases for PRS measurements with gaps in RRC_CONNECTED state	1989

A.3.34.4	Test cases for PRS measurements without gaps in RRC_CONNECTED state	1989

A.3.34.5	Testing principles for positioning measurements by aggregating PRS resources from multiple PFLs	1989

A.3.34.6	Testing principles for carrier phase measurement for positioning	1990

A.3.34.7	Test cases in RRC_IDLE state	1990

A.3.35	Testing principle for RedCap UE	1990

A.3.35.1	Introduction	1990

A.3.35.2	Principle of testing for FR1	1990

A.3.35.3	Principle of testing for FR2	1990

A.3.35.4	Principle of testing for PRS measurement	1990

A.3.36	Testing related to Satellite access	1991

A.3.36.1	Introduction	1991

A.3.36.2	Principle of testing GSO and NGSO scenarios	1991

A.3.36.3	Principle of testing different RRM requirements	1991

A.3.36.4	Principle of testing different ephemeris formats	1992

A.3.36.5	General setup for SIB19	1994

A.3.36.6	Satellite specific parameters configuration	1995

A.3.36.6.1	Satellite specific configuration for serving cell	1995

A.3.36.6.2	Satellite specific configuration for neighbour cell	1995

A.3.37	Reference Cell DTX configurations	1996

A.3.37.1	Cell DTX Configuration 1: Cell DTX cycle = 160 ms and TAT = Infinity	1996

A.3.38	DL-PRS Measurement Time Window configurations	1996

A.4	EN-DC tests with all NR cells in FR1	1998

A.4.1	Void	1998

A.4.2	Void	1998

A.4.3	RRC_CONNECTED state mobility	1998

A.4.3.1	Void	1998

A.4.3.2	RRC Connection Mobility Control	1998

A.4.3.2.1	Void	1998

A.4.3.2.2	Random Access	1998

A.4.3.2.2.1	4-step RA type contention based random access test in FR1 for PSCell in EN-DC	1998

A.4.3.2.2.2	4-step RA type n on-contention based random access test in FR1 for PSCell in EN-DC	2001

A.4.3.2.2.3	2-step RA type contention based random access test in FR1 for PSCell in EN-DC	2003

A.4.3.2.2.4	2-step RA type non-contention based random access test in FR1 for PSCell in EN-DC	2006

A.4.3.2.3	Void	2008

A.4.3.3	Handover with PSCell from EN-DC to EN-DC with known target PSCell in FR1	2008

A.4.3.3.1	Test Purpose and Environment	2008

A.4.3.3.2	Test Requirements	2011

A.4.4	Timing	2012

A.4.4.1	UE transmit timing	2012

A.4.4.1.1	NR UE Transmit Timing Test for FR1	2012

A.4.4.1.1.1	Test Purpose and environment	2012

A.4.4.1.1.2	Test requirements	2014

A.4.4.1.2	NR UE Transmit Timing Test for two TRPs in FR1	2015

A.4.4.1.2.1	Test Purpose and environment	2015

A.4.4.1.2.2	Test requirements	2018

A.4.4.2	UE timer accuracy	2019

A.4.4.3	Timing advance	2019

A.4.4.3.1	EN-DC FR1 timing advance adjustment accuracy	2019

A.4.4.3.1.1	Test Purpose and Environment	2019

A.4.4.3.1.2	Test Parameters	2019

A.4.4.3.1.3	Test Requirements	2022

A.4.5	Signaling characteristics	2022

A.4.5.1	Radio link Monitoring	2022

A.4.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	2022

A.4.5.1.1.1	Test Purpose and Environment	2022

A.4.5.1.1.2	Test Requirements	2026

A.4.5.1.2	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	2026

A.4.5.1.2.1	Test Purpose and Environment	2026

A.4.5.1.2.2	Test Requirements	2029

A.4.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	2029

A.4.5.1.3.1	Test Purpose and Environment	2029

A.4.5.1.3.2	Test Requirements	2032

A.4.5.1.4	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	2032

A.4.5.1.4.1	Test Purpose and Environment	2032

A.4.5.1.4.2	Test Requirements	2035

A.4.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	2035

A.4.5.1.5.1	Test Purpose and Environment	2035

A.4.5.1.5.2	Test Requirements	2038

A.4.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	2039

A.4.5.1.6.1	Test Purpose and Environment	2039

A.4.5.1.6.2	Test Requirements	2041

A.4.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	2042

A.4.5.1.7.1	Test Purpose and Environment	2042

A.4.5.1.7.2	Test Requirements	2044

A.4.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	2045

A.4.5.1.8.1	Test Purpose and Environment	2045

A.4.5.1.8.2	Test Requirements	2048

A.4.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	2048

A.4.5.1.9.1	Test Purpose and Environment	2048

A.4.5.1.10	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode when CD-SSB is outside active BWP	2051

A.4.5.1.10.1	Test Purpose and Environment	2051

A.4.5.1.11	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode when CD-SSB is outside active BWP	2051

A.4.5.1.11.1	Test Purpose and Environment	2051

A.4.5.1.11.2	Test Requirements	2052

A.4.5.1.12	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode for UE supporting NCD-SSB based measurement outside active BWP	2052

A.4.5.1.12.1	Test Purpose and Environment	2052

A.4.5.1.12.2	Test Requirements	2055

A.4.5.2	Interruption	2055

A.4.5.2.1	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	2055

A.4.5.2.1.1	Test Purpose and Environment	2055

A.4.5.2.1.2	Test Requirements	2057

A.4.5.2.2	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	2057

A.4.5.2.2.1	Test Purpose and Environment	2057

A.4.5.2.2.2	Test Requirements	2059

A.4.5.2.3	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	2059

A.4.5.2.3.1	Test Purpose and Environment	2059

A.4.5.2.3.2	Test Requirements	2063

A.4.5.2.4	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	2064

A.4.5.2.4.1	Test Purpose and Environment	2064

A.4.5.2.4.2	Test Requirements	2068

A.4.5.2.5	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	2068

A.4.5.2.5.1	Test Purpose and Environment	2068

A.4.5.2.5.2	Test Requirements	2070

A.4.5.2.6	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	2071

A.4.5.2.6.1	Test Purpose and Environment	2071

A.4.5.2.6.2	Test Requirements	2073

A.4.5.2.7	Void	2073

A.4.5.2.8	E-UTRAN - NR FR1 interruptions at NR SRS carrier based switching in asynchronous EN-DC	2073

A.4.5.2.8.1	Test Purpose and Environment	2073

A.4.5.2.8.2	Test Requirements	2076

A.4.5.2.9	E-UTRAN – NR interruptions at E-UTRA SRS carrier based switching	2076

A.4.5.2.9.1	Test Purpose and Environment	2076

A.4.5.2.9.2	Test Requirements	2079

A.4.5.2.10	E-UTRAN – NR FR1 interruptions due to RRM and RLM/BFD measurements on deactivated NR PSCell	2079

A.4.5.2.10.1	Test Purpose and Environment	2079

A.4.5.2.10.2	Test Requirements	2081

A.4.5.2.11	E-UTRAN - NR FR1 interruptions at NR SRS antenna port switching with 1 SRS symbol in a slot in synchronous EN-DC	2081

A.4.5.2.11.1	Test Purpose and Environment	2081

A.4.5.2.11.2	Test Requirements	2084

A.4.5.2.12	E-UTRAN - NR FR1 interruptions at NR SRS antenna port switching in asynchronous EN-DC	2084

A.4.5.2.12.1	Test Purpose and Environment	2084

A.4.5.3	SCell Activation and Deactivation Delay	2088

A.4.5.3.1	SCell Activation and deactivation of known SCell in FR1 for 160 ms SCell measurement cycle	2088

A.4.5.3.1.1	Test Purpose and Environment	2088

A.4.5.3.1.2	Test Requirements	2094

A.4.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 640 ms SCell measurement cycle	2094

A.4.5.3.2.1	Test Purpose and Environment	2094

A.4.5.3.2.2	Test Requirements	2094

A.4.5.3.3	SCell Activation and deactivation of unknown SCell in FR1	2095

A.4.5.3.3.1	Test Purpose and Environment	2095

A.4.5.3.3.2	Test Requirements	2095

A.4.5.3.4	SCell Activation and deactivation of multiple unknown SCells in FR1 with single activation/deactivation command	2095

A.4.5.3.4.1	Test Purpose and Environment	2096

A.4.5.3.4.2	Test Requirements	2098

A.4.5.3.5	Direct SCell activation at SCell addition of known SCell in FR1	2098

A.4.5.3.5.1	Test Purpose and Environment	2098

A.4.5.3.5.2	Test Requirements	2103

A.4.5.3.6	Fast SCell Activation of known SCell in FR1 for 160 ms SCell measurement cycle	2103

A.4.5.3.6.1	Test Purpose and Environment	2103

A.4.5.3.6.2	Test Requirements	2106

A.4.5.3.7	Fast SCell Activation of known SCell in FR1 for 640 ms SCell measurement cycle	2107

A.4.5.3.7.1	Test Purpose and Environment	2107

A.4.5.3.7.2	Test Requirements	2107

A.4.5.3.8	SCell Activation and deactivation of unknown SCell in FR1 for UE capable of short measurement interval	2107

A.4.5.3.8.1	Test Purpose and Environment	2107

A.4.5.3.8.2	Test Requirements	2108

A.4.5.3.9	SCell Activation of unknown SCell with valid L3 measurement results in FR1 for 160 ms SCell measurement cycle	2109

A.4.5.3.9.1	Test Purpose and Environment	2109

A.4.5.3.9.2	Test Requirements	2114

A.4.5.3.10	SCell Activation of multiple unknown SCells in FR1 with L3 reporting with single activation/deactivation command in non-DRX	2115

A.4.5.3.10.1	Test Purpose and Environment	2115

A.4.5.3.10.2	Test Requirements	2117

A.4.5.3.11	TRS-based SCell Activation of SSB-less SCell in FR1 collocated inter-band	2118

A.4.5.3.11.1	Test Purpose and Environment	2118

A.4.5.3.11.2	Test Requirements	2121

A.4.5.3.12	Inter-band SSB-less Scell activation using A-TRS	2122

A.4.5.3.12.1	Test Purpose and Environment	2122

A.4.5.3.12.2	Test Requirements	2125

A.4.5.4	UE UL carrier RRC reconfiguration Delay	2125

A.4.5.4.1	UE UL carrier RRC reconfiguration Delay	2125

A.4.5.4.1.1 Test Purpose and Environment	2125

A.4.5.4.1.2	Test Requirements	2130

A.4.5.5	Beam Failure Detection and Link recovery procedures	2130

A.4.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	2130

A.4.5.5.1.1	Test Purpose and Environment	2130

A.4.5.5.1.2	Test Requirements	2134

A.4.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	2134

A.4.5.5.2.1	Test Purpose and Environment	2134

A.4.5.5.2.2	Test Requirements	2137

A.4.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	2138

A.4.5.5.3.1	Test Purpose and Environment	2138

A.4.5.5.3.2	Test Requirements	2141

A.4.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in DRX mode	2142

A.4.5.5.4.1	Test Purpose and Environment	2142

A.4.5.5.4.2	Test Requirements	2145

A.4.5.5.5	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	2146

A.4.5.5.5.1	Test Purpose and Environment	2146

A.4.5.5.5.2	Test Requirements	2149

A.4.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	2150

A.4.5.5.6.1	Test Purpose and Environment	2150

A.4.5.5.6.2	Test Requirements	2153

A.4.5.5.7	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	2154

A.4.5.5.7.1	Test Purpose and Environment	2154

A.4.5.5.7.2	Test Requirements	2157

A.4.5.5.8	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	2158

A.4.5.5.8.1	Test Purpose and Environment	2158

A.4.5.5.8.2	Test Requirements	2162

A.4.5.6	Active BWP switch	2162

A.4.5.6.1	DCI-based and Timer-based Active BWP Switch	2162

A.4.5.6.1.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	2162

A.4.5.6.1.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	2166

A.4.5.6.2	RRC-based Active BWP Switch	2171

A.4.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	2174

A.4.5.6.3.1	Simultaneous E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in EN-DC on multiple CCs	2174

A.4.5.6.4	Simultaneous RRC-based Active BWP Switch on multiple CCs	2179

A.4.5.6.4.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC on multiple CCs	2179

A.4.5.6.4.1.1	Test Purpose and Environment	2179

A.4.5.6.4.1.2	Test Requirements	2183

A.4.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR1 SCells inside active time	2183

A.4.5.6.4.2.1	Test Purpose and Environment	2183

A.4.5.6.4.2.2	Test Requirements	2189

A.4.5.6.5	SCell dormancy switch	2189

A.4.5.6.5.1	E-UTRAN – NR FR1 PSCell SCell dormancy switch of single FR1 SCell outside active time	2189

A.4.5.6.5.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR1 SCells inside active time	2194

A.4.5.6.5.2.1	Test Purpose and Environment	2194

A.4.5.6.5.2.2	Test Requirements	2198

A.4.5.7	PSCell addition and release delay	2198

A.4.5.7.1	Addition and Release Delay of known NR PSCell	2198

A.4.5.7.1.1	Test purpose and environment	2198

A.4.5.7.1.2	Test Requirements	2201

A.4.5.8	DL Interruptions at switching between two uplink carriers	2201

A.4.5.8.1	Test Purpose and Environment	2201

A.4.5.8.2	Test Requirements	2205

A.4.5.9	UE specific CBW change	2205

A.4.5.9.1	UE specific CBW change on FR1 NR PSCell with non-DRX in synchronous EN- DC	2205

A.4.5.9.1.1	Test Purpose and Environment	2205

A.4.5.9.1.2	Test Requirements	2208

A.4.5.10	PSCell activation and deactivation delay	2208

A.4.5.10.1	PSCell activation and deactivation delay	2208

A.4.5.10.1.1	Test purpose and environment	2208

A.4.5.10.1.2	Test Requirements	2210

A.4.5.11	Conditional PSCell addition and release delay (FR1 EN-DC)	2211

A.4.5.11.1	Conditional PSCell Addition and Release Delay	2211

A.4.5.11.1.1	Test purpose and environment	2211

A.4.5.11.1.2	Test Parameters	2211

A.4.5.11.1.3	Test Requirements	2213

A.4.6	Measurement procedure	2213

A.4.6.1	Intra-frequency Measurements	2213

A.4.6.1.1	EN-DC event triggered reporting tests without gap under non-DRX	2213

A.4.6.1.1.1	Test purpose and Environment	2213

A.4.6.1.1.2	Test parameters	2214

A.4.6.1.1.3	Test Requirements	2215

A.4.6.1.2	EN-DC event triggered reporting tests without gap under DRX	2215

A.4.6.1.2.1	Test purpose and Environment	2216

A.4.6.1.2.2	Test parameters	2216

A.4.6.1.2.2	Test Requirements	2218

A.4.6.1.3	EN-DC event triggered reporting tests with per-UE gaps under non-DRX	2218

A.4.6.1.3.1	Test purpose and Environment	2218

A.4.6.1.3.2	Test parameters	2218

A.4.6.1.3.3	Test Requirements	2220

A.4.6.1.4	EN-DC event triggered reporting tests with per-UE gaps under DRX	2220

A.4.6.1.4.1	Test purpose and Environment	2220

A.4.6.1.4.2	Test parameters	2220

A.4.6.1.4.3	Test Requirements	2222

A.4.6.1.5	EN-DC event triggered reporting tests without gap under non-DRX with SSB index reading	2223

A.4.6.1.5.1	Test purpose and Environment	2223

A.4.6.1.5.2	Test parameters	2223

A.4.6.1.5.3	Test Requirements	2224

A.4.6.1.6	EN-DC event triggered reporting tests with SSB index reading with per-UE gaps	2224

A.4.6.1.6.1	Test purpose and Environment	2224

A.4.6.1.6.2	Test parameters	2224

A.4.6.1.6.3	Test Requirements	2226

A.4.6.1.7	EN-DC event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	2226

A.4.6.1.7.1	Test purpose and Environment	2226

A.4.6.1.7.2	Test parameters	2226

A.4.6.1.7.3	Test Requirements	2228

A.4.6.1.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used for UE configured with highSpeedMeasCA-Scell-r17	2228

A.4.6.1.8.1	Test Purpose and Environment	2228

A.4.6.1.8.2	Test Requirements	2232

A.4.6.1.9	EN-DC event triggered reporting tests without gap under non-DRX with NCD-SSB	2232

A.4.6.1.9.1	Test purpose and Environment	2232

A.4.6.1.9.2	Test parameters	2232

A.4.6.1.9.3	Test Requirements	2234

A.4.6.1.10	EN-DC event triggered reporting tests without gap under non-DRX when CD-SSB is outside active BWP	2234

A.4.6.110.1	Test purpose and Environment	2234

A.4.6.1.10.2	Test Requirements	2234

A.4.6.2	Inter-frequency Measurements	2234

A.4.6.2.1	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	2235

A.4.6.2.1.1	Test Purpose and Environment	2235

A.4.6.2.1.2	Test Requirements	2237

A.4.6.2.2	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	2237

A.4.6.2.2.1	Test Purpose and Environment	2237

A.4.6.2.2.2	Test Requirements	2240

A.4.6.2.3	Void	2240

A.4.6.2.4	Void	2240

A.4.6.2.5	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	2240

A.4.6.2.5.1	Test Purpose and Environment	2240

A.4.6.2.5.2	Test Requirements	2243

A.4.6.2.6	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	2243

A.4.6.2.6.1	Test Purpose and Environment	2243

A.4.6.2.6.2	Test Requirements	2245

A.4.6.2.7	Void	2246

A.4.6.2.8	Void	2246

A.4.6.2.9	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used for UE configured with highSpeedMeasInterFreq-r17	2246

A.4.6.2.9.1	Test Purpose and Environment	2246

A.4.6.2.9.2	Test Requirements	2249

A.4.6.3	Void	2249

A.4.6.4	L1-RSRP measurement for beam reporting	2249

A.4.6.4.1	SSB based L1-RSRP measurement when DRX is not used	2249

A.4.6.4.1.1	Test Purpose and Environment	2249

A.4.6.4.1.2	Test parameters	2249

A.4.6.4.1.3	Test Requirements	2250

A.4.6.4.2	SSB based L1-RSRP measurement when DRX is used	2251

A.4.6.4.2.1	Test Purpose and Environment	2251

A.4.6.4.2.2	Test parameters	2251

A.4.6.4.2.3	Test Requirements	2252

A.4.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	2253

A.4.6.4.3.1	Test Purpose and Environment	2253

A.4.6.4.3.2	Test parameters	2253

A.4.6.4.3.3	Test Requirements	2254

A.4.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	2255

A.4.6.4.4.1	Test Purpose and Environment	2255

A.4.6.4.4.2	Test parameters	2255

A.4.6.4.4.3	Test Requirements	2256

A.4.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	2257

A.4.6.4.5.1	Test Purpose and Environment	2257

A.4.6.4.5.2	Test parameters	2257

A.4.6.4.5.3	Test Requirements	2258

A.4.6.4.6	CSI-RS based L1-RSRP measurement when DRX is not used when CD-SSB is outside active BWP	2258

A.4.6.4.6.1	Test Purpose and Environment	2259

A.4.6.4.7	SSB based L1-RSRP measurement when DRX is not used when CD-SSB is outside active BWP	2259

A.4.6.4.7.1	Test Purpose and Environment	2259

A.4.6.4.7.2	Test Requirements	2259

A.4.6.4.8	SSB based L1-RSRP measurement for UE supporting NCD-SSB based L1 measurement outside active BWP when DRX is not used	2259

A.4.6.4.8.1	Test Purpose and Environment	2259

A.4.6.4.8.2	Test parameters	2259

A.4.6.4.8.3	Test Requirements	2261

A.4.6.5	CLI measurements	2261

A.4.6.5.1	SRS-RSRP measurement with non-DRX	2261

A.4.6.5.1.1	Test Purpose and Environment	2261

A.4.6.5.1.2	Test Parameters	2261

A.4.6.5.1.3	Test Requirements	2264

A.4.6.5.2	CLI-RSSI measurement with non-DRX	2264

A.4.6.5.2.1	Test Purpose and Environment	2264

A.4.6.5.2.2	Test Parameters	2264

A.4.6.5.2.3	Test Requirements	2265

A.4.6.6.1.2	Test Requirements	2269

A.4.6.7	L1-SINR measurement for beam reporting	2269

A.4.6.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	2271

A.4.6.7.2.1	Test Purpose and Environment	2271

A.4.6.7.2.2	Test parameters	2272

A.4.6.7.2.3	Test Requirements	2273

A.4.6.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	2273

A.4.6.7.3.1	Test Purpose and Environment	2274

A.4.6.7.3.2	Test parameters	2274

A.4.6.7.3.3	Test Requirements	2275

A.4.6.8	CSI-RS based intra-frequency Measurement	2276

A.4.6.8.1	EN-DC event triggered reporting tests without gap under DRX	2276

A.4.6.8.1.1	Test purpose and Environment	2276

A.4.6.8.1.2	Test Requirements	2278

A.4.6.9	CSI-RS based inter-frequency Measurement	2278

A.4.6.9.1	EN-DC event triggered reporting tests for FR1 cell when non-DRX is used	2278

A.4.6.9.1.1	Test Purpose and Environment	2278

A.4.6.9.1.2	Test Requirements	2280

A.6	NR standalone tests with all NR cells in FR1	2282

A.6.1	SA: RRC_IDLE state mobility	2282

A.6.1.1	Cell re-selection to NR	2282

A.6.1.1.1	Cell reselection to FR1 intra-frequency NR case	2282

A.6.1.1.1.1	Test Purpose and Environment	2282

A.6.1.1.1.2	Test Parameters	2282

A.6.1.1.1.3	Test Requirements	2284

A.6.1.1.2	Cell reselection to FR1 inter-frequency NR case	2284

A.6.1.1.2.1	Test Purpose and Environment	2284

A.6.1.1.2.2	Test Parameters	2285

A.6.1.1.2.3	Test Requirements	2287

A.6.1.1.3	Cell reselection to FR1 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	2287

A.6.1.1.3.1	Test Purpose and Environment	2287

A.6.1.1.3.2	Test Parameters	2287

A.6.1.1.3.3	Test Requirements	2290

A.6.1.1.4	Cell reselection to FR1 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	2290

A.6.1.1.4.1	Test Purpose and Environment	2290

A.6.1.1.4.2	Test Parameters	2290

A.6.1.1.4.3	Test Requirements	2292

A.6.1.1.5	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	2293

A.6.1.1.5.1	Test Purpose and Environment	2293

A.6.1.1.5.2	Test Parameters	2293

A.6.1.1.5.3	Test Requirements	2295

A.6.1.1.6	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	2296

A.6.1.1.6.1	Test Purpose and Environment	2296

A.6.1.1.6.2	Test Parameters	2296

A.6.1.1.6.3	Test Requirements	2298

A.6.1.1.7	Cell reselection to FR1 intra-frequency NR case for UE configured with highSpeedMeasFlag-r16	2299

A.6.1.1.7.1	Test Purpose and Environment	2299

A.6.1.1.7.2	Test Parameters	2299

A.6.1.1.7.3	Test Requirements	2302

A.6.1.1.8	Cell reselection to FR1 inter-frequency NR case for UE configured with highSpeedMeasInterFreq-r17	2302

A.6.1.1.8.1	Test Purpose and Environment	2302

A.6.1.1.8.2	Test Parameters	2302

A.6.1.1.8.3	Test Requirements	2305

A.6.1.1.9	Cell reselection to FR1 intra-frequency NR case for UE operating on a cell with less than 5 MHz BW	2305

A.6.1.1.9.1	Test Purpose and Environment	2305

A.6.1.1.9.3	Test Requirements	2306

A.6.1.2	Inter-RAT E-UTRAN cell re-selection	2306

A.6.1.2.1	Cell reselection to higher priority E-UTRAN	2306

A.6.1.2.1.1	Test Purpose and Environment	2306

A.6.1.2.1.2	Test Parameters	2307

A.6.1.2.1.3	Test Requirements	2309

A.6.1.2.2	Cell reselection to lower priority E-UTRAN	2309

A.6.1.2.2.1	Test Purpose and Environment	2309

A.6.1.2.2.2	Test Parameters	2309

A.6.1.2.2.3	Test Requirements	2312

A.6.1.2.3	Cell reselection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion	2312

A.6.1.2.3.1	Test Purpose and Environment	2312

A.6.1.2.3.2	Test Parameters	2312

A.6.1.2.3.3	Test Requirements	2315

A.6.1.2.4	Cell reselection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion	2315

A.6.1.2.4.1	Test Purpose and Environment	2315

A.6.1.2.4.2	Test Parameters	2316

A.6.1.2.4.3	Test Requirements	2318

A.6.1.2.5	Cell reselection to lower priority E-UTRAN cell for UE configured with highSpeedMeasFlag-r16	2318

A.6.1.2.5.1	Test Purpose and Environment	2319

A.6.1.2.5.2	Test Parameters	2319

A.6.1.2.5.3	Test Requirements	2321

A.6.1.1.7	Void	2321

A.6.2	SA: RRC_INACTIVE state mobility	2322

A.6.2.1	Configured Grant based Small Data Transmissions (CG-SDT)	2322

A.6.2.1.1	Test purpose and Environment	2322

A.6.2.1.2	Test Parameters	2323

A.6.2.1.3	Test requirements	2324

A.6.2.2	Cell reselection for positioning	2325

A.6.2.2.1	Cell reselection to FR1 intra-frequency NR case with RRC_ INACTIVE eDRX and positioning SRS	2325

A.6.2.2.1.1	Test Purpose and Environment	2325

A.6.2.2.1.2	Test Parameters	2325

A.6.2.2.1.3	Test Requirements	2329

A.6.3	RRC_CONNECTED state mobility	2329

A.6.3.1	Handover	2329

A.6.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell	2329

A.6.3.1.1.1	Test Purpose and Environment	2329

A.6.3.1.1.2	Test Parameters	2329

A.6.3.1.1.3 Test Requirements	2331

A.6.3.1.2	Intra-frequency handover from FR1 to FR1; unknown target cell	2331

A.6.3.1.2.1	Test Purpose and Environment	2331

A.6.3.1.2.2	Test Parameters	2331

A.6.3.1.2.3	Test Requirements	2333

A.6.3.1.3	Inter-frequency handover from FR1 to FR1; unknown target cell	2333

A.6.3.1.3.1	Test Purpose and Environment	2333

A.6.3.1.3.2	Test Parameters	2333

A.6.3.1.3.3	Test Requirements	2335

A.6.3.1.4	SA NR - E-UTRAN handover	2335

A.6.3.1.4.1	Test Purpose and Environment	2335

A.6.3.1.4.2	Test Requirements	2338

A.6.3.1.5	SA NR - E-UTRAN handover with unknown target cell	2339

A.6.3.1.5.1	Test Purpose and Environment	2339

A.6.3.1.5.2	Test Requirements	2342

A.6.3.1.6	 SA NR - UTRAN FDD handover	2342

A.6.3.1.6.1	Test Purpose and Environment	2342

A.6.3.1.6.2	Test Requirements	2344

A.6.3.1.7	Intra-frequency synchronous DAPS handover in FR1	2344

A.6.3.1.7.1	Test Purpose and Environment	2344

A.6.3.1.7.2	Test Parameters	2344

A.6.3.1.7.3	Test Requirements	2347

A.6.3.1.8	Intra-frequency asynchronous DAPS handover in FR1	2347

A.6.3.1.8.1	Test Purpose and Environment	2347

A.6.3.1.8.2	Test Parameters	2348

A.6.3.1.8.3	Test Requirements	2350

A.6.3.1.9	Intra-band inter-frequency synchronous DAPS handover test in SA for FR1	2350

A.6.3.1.9.1	Test Purpose and Environment	2350

A.6.3.1.9.2	Test Parameters	2351

A.6.3.1.9.3	Test Requirements	2353

A.6.3.1.10	Intra-band inter-frequency asynchronous DAPS handover test in SA for FR1	2353

A.6.3.1.10.1	Test Purpose and Environment	2353

A.6.3.1.10.2	Test Parameters	2353

A.6.3.1.10.3	Test Requirements	2355

A.6.3.1.11	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR1	2355

A.6.3.1.11.1	Test Purpose and Environment	2355

A.6.3.1.11.2	Test Parameters	2355

A.6.3.1.11.3 Test Requirements	2359

A.6.3.1.12	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR1	2360

A.6.3.1.12.1	Test Purpose and Environment	2360

A.6.3.1.12.2	Test Parameters	2360

A.6.3.1.12.3	Test Requirements	2364

A.6.3.1.13	SA NR - E-UTRAN with NR PSCell addition in FR1	2364

A.6.3.1.13.1	Test Purpose and Environment	2364

A.6.3.1.13.2	Test Requirements	2369

A.6.3.1.14	SA NR - E-UTRAN handover with NR FR1 PSCell addition	2369

A.6.3.1.14.1	Test Purpose and Environment	2369

A.6.3.1.14.2	Test Requirements	2374

A.6.3.1.15	Intra-frequency handover from FR1 to FR1; known target cell configured with NCD-SSB	2375

A.6.3.1.15.1	Test Purpose and Environment	2375

A.6.3.1.15.2	Test Parameters	2375

A.6.3.1.15.3	Test Requirements	2378

A.6.3.1.16	Inter-frequency handover from FR1 to FR1; known target cell configured with NCD-SSB	2378

A.6.3.1.16.1	Test Purpose and Environment	2378

A.6.3.1.16.2	Test Parameters	2378

A.6.3.1.16.3 Test Requirements	2380

A.6.3.1.17	Handover with PSCell change delay from NR-DC (FR1-FR1) to NR-DC (FR1-FR1)	2380

A.6.3.1.17.1	Test Purpose and Environment	2380

A.6.3.1.17.2	Test Requirements	2384

A.6.3.1.18	Intra-frequency handover from FR1 to FR1; unknown target cell operating with 12 PRB SSB bandwidth	2384

A.6.3.1.18.2	Test Parameters	2384

A.6.3.1.18.3	Test Requirements	2385

A.6.3.1.19	Handover with PSCell change delay where target PSCell is with 12PRB SSB bandwidth	2386

A.6.3.1.19.1	Test Purpose and Environment	2386

A.6.3.1.19.2	Test Parameters	2386

A.6.3.1.19.3	Test Requirements	2388

A.6.3.2	RRC Connection Mobility Control	2389

A.6.3.2.1	SA: RRC Re-establishment	2389

A.6.3.2.1.1	Intra-frequency RRC Re-establishment in FR1	2389

A.6.3.2.1.2	Inter-frequency RRC Re-establishment in FR1	2392

A.6.3.2.1.3	Intra-frequency RRC Re-establishment in FR1 without serving cell timing	2394

A.6.3.2.2	Random Access	2397

A.6.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	2397

A.6.3.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone	2400

A.6.3.2.2.3	2-step RA type contention based random access test in FR1 for NR standalone	2403

A.6.3.2.2.4	2-step RA type non-contention based test in FR1 for NR standalone	2405

A.6.3.2.3	SA: RRC Connection Release with Redirection	2408

A.6.3.2.3.1	Redirection from NR in FR1 to NR in FR1	2408

A.6.3.2.3.2	Redirection from NR in FR1 to E-UTRAN	2410

A.6.3.2.4	LTM PDCCH-order Random Access	2413

A.6.3.2.4.1	PDCCH-order RACH on neighbor cell in FR1 when RACH BW is within active UL BWP	2413

A.6.3.2.4.2	PDCCH-ordered RACH to an inter-frequency candidate cell in FR1 for LTM	2417

A.6.3.2.4.3	PDCCH-order RACH on neighbor cell without L1-RSRP measurement in FR1 when RACH BW is within active UL BWP	2421

A.6.3.3	Conditional handover	2424

A.6.3.3.1	Intra-frequency conditional handover from FR1 to FR1	2424

A.6.3.3.1.1	Test Purpose and Environment	2424

A.6.3.3.1.2	Test Parameters	2424

A.6.3.3.1.3	Test Requirements	2426

A.6.3.3.2	Inter-frequency conditional handover from FR1 to FR1	2426

A.6.3.3.2.1	Test Purpose and Environment	2426

A.6.3.3.2.2	Test Parameters	2426

A.6.3.3.2.3	Test Requirements	2428

A.6.3.3.3	NR conditional handover including target MCG and target SCG from FR1-FR1 NR-DC to FR1-FR1 NR-DC	2428

A.6.3.3.3.1	Test Purpose and Environment	2428

A.6.3.3.3.2	Test Requirements	2431

A.6.3.3.4	NR conditional handover including target MCG and candidate SCG from FR1-FR1 NR-DC to FR1-FR1 NR-DC	2432

A.6.3.3.4.1	Test Purpose and Environment	2432

A.6.3.3.4.2	Test Parameters	2432

A.6.3.3.4.3	Test Requirements	2436

A.6.3.3.5	NR conditional handover including target MCG and candidate SCG from FR1-FR1 NR-DC to FR1-FR1 NR-DC with complementary conditional handover configuration	2436

A.6.3.3.5.1	Test Purpose and Environment	2436

A.6.3.3.5.2	Test Parameters	2436

A.6.3.3.5.3	Test Requirements	2440

A.6.3.3.6	NES triggering intra-frequency conditional handover from FR1 to FR1	2440

A.6.3.3.6.1	Test Purpose and Environment	2440

A.6.3.3.6.2	Test Parameters	2440

A.6.3.3.6.3	Test Requirements	2442

A.6.3.3.7	NES-based Inter-frequency conditional handover from FR1 to FR1	2442

A.6.3.3.7.1	Test Purpose and Environment	2442

A.6.3.3.7.2	Test Parameters	2442

A.6.3.3.7.3	Test Requirements	2444

A.6.3.4	LTM PCell Switch	2444

A.6.3.4.1	RACH-based Intra-frequency PCell switch from FR1 to FR1	2444

A.6.3.4.1.1	Test Purpose and Environment	2444

A.6.3.4.1.2	Test Parameters	2444

A.6.3.4.1.3	Test Requirements	2447

A.6.3.4.2	RACH based Inter-frequency LTM PCell switch from FR1 to FR1	2448

A.6.3.4.2.1	Test Purpose and Environment	2448

A.6.3.4.2.2	Test Parameters	2448

A.6.3.4.2.3	Test Requirements	2451

A.6.3.4.3	RACH-less Intra-frequency PCell switch from FR1 to FR1	2452

A.6.3.4.3.1	Test Purpose and Environment	2452

A.6.3.4.3.2	Test Parameters	2452

A.6.3.4.3.3	Test Requirements	2456

A.6.3.4.4	RACH-less Intra-frequency PCell switch from FR1 to FR1 without L1-RSRP measurement	2456

A.6.3.4.4.1	Test Purpose and Environment	2456

A.6.3.4.4.2	Test Parameters	2456

A.6.3.4.4.3	Test Requirements	2460

A.6.3.5	LTM PSCell Switch	2460

A.6.3.5.1 RACH-based intra-frequency LTM PSCell switch from FR1 to FR1	2460

A.6.3.5.1.1	Test Purpose and Environment	2460

A.6.3.5.1.2	Test Parameters	2460

A.6.3.5.1.3	Test Requirements	2465

A.6.4	Timing	2465

A.6.4.1	UE transmit timing	2465

A.6.4.1.1	NR UE Transmit Timing Test for FR1	2465

A.6.4.1.1.1	Test Purpose and environment	2465

A.6.4.1.1.2	Test requirements	2467

A.6.4.1.2	NR UE Transmit Timing Test for two TRPs in FR1	2468

A.6.4.1.2.1	Test Purpose and environment	2468

A.6.4.1.2.2	Test requirements	2470

A.6.4.2	UE timer accuracy	2471

A.6.4.3	Timing advance	2471

A.6.4.3.1	SA FR1 timing advance adjustment accuracy	2471

A.6.4.3.1.1	Test Purpose and Environment	2471

A.6.4.3.1.2	Test Parameters	2471

A.6.4.3.1.3	Test Requirements	2474

A.6.5	Signalling characteristics	2474

A.6.5.1	Radio link Monitoring	2474

A.6.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	2474

A.6.5.1.1.1	Test Purpose and Environment	2474

A.6.5.1.1.2	Test Requirements	2477

A.6.5.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	2477

A.6.5.1.2.1	Test Purpose and Environment	2477

A.6.5.1.2.2	Test Requirements	2480

A.6.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	2480

A.6.5.1.3.1	Test Purpose and Environment	2480

A.6.5.1.3.2	Test Requirements	2483

A.6.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	2483

A.6.5.1.4.1	Test Purpose and Environment	2483

A.6.5.1.4.2	Test Requirements	2486

A.6.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	2486

A.6.5.1.5.1	Test Purpose and Environment	2486

A.6.5.1.5.2	Test Requirements	2489

A.6.5.1.6	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	2489

A.6.5.1.6.1	Test Purpose and Environment	2489

A.6.5.1.6.2	Test Requirements	2492

A.6.5.1.7	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	2492

A.6.5.1.7.1	Test Purpose and Environment	2492

A.6.5.1.7.2	Test Requirements	2495

A.6.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	2496

A.6.5.1.8.1	Test Purpose and Environment	2496

A.6.5.1.8.2	Test Requirements	2499

A.6.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM for UE fulfilling relaxed measurement criterion	2499

A.6.5.1.9.1	Test Purpose and Environment	2499

A.6.5.1.9.2	Test Requirements	2502

A.6.5.1.10	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode when CD-SSB is outside active BWP	2502

A.6.5.1.10.1	Test Purpose and Environment	2502

A.6.5.1.11	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode when CD-SSB is outside active BWP	2503

A.6.5.1.11.1	Test Purpose and Environment	2503

A.6.5.1.11.2	Test Requirements	2503

A.6.5.1.12	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for UE supporting NCD-SSB based measurement outside active BWP	2503

A.6.5.1.12.1	Test Purpose and Environment	2503

A.6.5.1.12.2	Test Requirements	2506

A.6.5.1.13	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for UE operating on a cell with less than 5 MHz BW	2506

A.6.5.1.13.1	Test Purpose and Environment	2506

A.6.5.1.13.2	Test Requirements	2507

A.6.5.1.14	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for UE operating on a cell with less than 5 MHz BW	2507

A.6.5.1.14.1	Test Purpose and Environment	2507

A.6.5.1.14.2	Test Requirements	2508

A.6.5.1.15	Radio Link Monitoring In-sync Test for FR1 PCell with 3 MHz Channel Bandwidth configured with SSB-based RLM RS in non-DRX mode	2508

A.6.5.1.15.1	Test Purpose and Environment	2508

A.6.5.1.15.2	Test Requirements	2509

A.6.5.1.16	Radio Link Monitoring In-sync Test for FR1 PCell with 3MHz Channel Bandwidth configured with SSB-based RLM RS in DRX mode	2509

A.6.5.1.16.1	Test Purpose and Environment	2509

A.6.5.1.16.2	Test Requirements	2510

A.6.5.2	Interruption	2511

A.6.5.2.1	Interruptions during measurements on deactivated NR SCC in FR1	2511

A.6.5.2.1.1	Test Purpose and Environment	2511

A.6.5.2.1.2	Test Requirements	2514

A.6.5.2.2	SA interruptions at NR SRS carrier based switching	2515

A.6.5.2.2.1	Test Purpose and Environment	2515

A.6.5.2.2.2	Test Parameters	2515

A.6.5.2.2.3	Test Requirements	2517

A.6.5.2.3	SA interruptions at NR SRS antenna port switching with 1 SRS symbol in a slot in NR-CA	2517

A.6.5.2.3.1	Test Purpose and Environment	2517

A.6.5.2.3.2	Test Parameters	2517

A.6.5.2.3.3	Test Requirements	2519

A.6.5.2.4	SA interruptions at NR SRS antenna port switching	2519

A.6.5.2.4.1	Test Purpose and Environment	2519

A.6.5.2.4.2	Test Parameters	2519

A.6.5.2.4.3	Test Requirements	2521

A.6.5.3	SCell Activation and Deactivation Delay	2521

A.6.5.3.1	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle	2521

A.6.5.3.1.1	Test Purpose and Environment	2521

A.6.5.3.1.2	Test Requirements	2526

A.6.5.3.2	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle	2526

A.6.5.3.2.1	Test Purpose and Environment	2526

A.6.5.3.2.2	Test Requirements	2527

A.6.5.3.3	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	2527

A.6.5.3.3.1	Test Purpose and Environment	2527

A.6.5.3.3.2	Test Requirements	2528

A.6.5.3.4	Direct SCell activation at SCell addition of known SCell in FR1	2528

A.6.5.3.4.1	Test Purpose and Environment	2528

A.6.5.3.4.2	Test Requirements	2531

A.6.5.3.5	Direct SCell activation at handover with known SCell in FR1	2532

A.6.5.3.5.1	Test Purpose and Environment	2532

A.6.5.3.5.2	Test Requirements	2536

A.6.5.3.6	PUCCH SCell Activation and deactivation of known SCell in FR1	2537

A.6.5.3.6.1	Test Purpose and Environment	2537

A.6.5.3.6.2	Test Requirements	2540

A.6.5.3.7	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	2540

A.6.5.3.7.1	Test Purpose and Environment	2540

A.6.5.3.7.2	Test Requirements	2543

A.6.5.3.8	SCell Activation and Deactivation of one FR1 known PUCCH SCell and one FR1 unknown SCell with single activation/deactivation command	2544

A.6.5.3.8.1	Test Purpose and Environment	2544

A.6.5.3.8.2	Test Requirements	2547

A.6.5.3.9	SCell Activation and deactivation of unknown PUCCH SCell and unknown DL SCell in FR1 in non-DRX	2548

A.6.5.3.9.1	Test Purpose and Environment	2548

A.6.5.3.9.2	Test Requirements	2551

A.6.5.3.10	Fast SCell Activation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle	2551

A.6.5.3.10.1	Test Purpose and Environment	2551

A.6.5.3.10.2	Test Requirements	2554

A.6.5.3.11	SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle	2555

A.6.5.3.11.1	Test Purpose and Environment	2555

A.6.5.3.11.2	Test Requirements	2555

A.6.5.3.12	SCell Activation and deactivation of unknown SCell in FR1 in DRX for UE capable of short measurement interval	2555

A.6.5.3.12.1	Test Purpose and Environment	2555

A.6.5.3.12.2	Test Requirements	2558

A.6.5.3.13	SCell Activation of multiple unknown SCells in FR1 with L3 reporting with single activation/deactivation commandin non-DRX	2558

A.6.5.3.13.1	Test Purpose and Environment	2558

A.6.5.3.13.2	Test Requirements	2563

A.6.5.3.14	SCell Activation of unknown SCell with valid L3 measurement results in FR1 in non-DRX for 160 ms SCell measurement cycle	2563

A.6.5.3.14.1	Test Purpose and Environment	2563

A.6.5.3.14.2	Test Requirements	2568

A.6.5.3.15	TRS based SCell Activation of SSB-less SCell in FR1 inter-band CA in non-DRX	2569

A.6.5.3.15.1	Test Purpose and Environment	2569

A.6.5.3.15.2	Test Requirements	2573

A.6.5.3.16	Inter-band SSB-less SCell Activation based on A-TRS	2573

A.6.5.3.16.1	Test Purpose and Environment	2573

A.6.5.3.16.2	Test Requirements	2577

A.6.5.3.17.1	Test Purpose and Environment	2578

A.6.5.3.17.2	Test Requirements	2579

A.6.5.4	UE UL carrier RRC reconfiguration Delay	2579

A.6.5.4.1	UE UL carrier RRC reconfiguration Delay	2579

A.6.5.4.1.1	Test Purpose and Environment	2579

A.6.5.4.1.2	Test Requirements	2582

A.6.5.4.2	Void	2582

A.6.5.5	Beam Failure Detection and Link recovery procedures	2582

A.6.5.5.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	2582

A.6.5.5.1.1	Test Purpose and Environment	2582

A.6.5.5.1.2	Test Requirements	2586

A.6.5.5.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	2586

A.6.5.5.2.1	Test Purpose and Environment	2586

A.6.5.5.2.2	Test Requirements	2590

A.6.5.5.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	2590

A.6.5.5.3.1	Test Purpose and Environment	2590

A.6.5.5.3.2	Test Requirements	2593

A.6.5.5.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	2594

A.6.5.5.4.1	Test Purpose and Environment	2594

A.6.5.5.4.2	Test Requirements	2598

A.6.5.5.5	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	2598

A.6.5.5.5.1	Test Purpose and Environment	2598

A.6.5.5.5.2	Test Requirements	2601

A.6.5.5.6	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	2602

A.6.5.5.6.1	Test Purpose and Environment	2602

A.6.5.5.6.2	Test Requirements	2605

A.6.5.5.7	TRP Specific Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	2606

A.6.5.5.7.1	Test Purpose and Environment	2606

A.6.5.5.7.2	Test Requirements	2609

A.6.5.5.8	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for a UE operating on a cell with less than 5 MHz BW	2610

A.6.5.5.8.1	Test Purpose and Environment	2610

A.6.5.5.8.2	Test Requirements	2611

A.6.5.6	Active BWP switch	2611

A.6.5.6.1	DCI-based and Timer-based Active BWP Switch	2611

A.6.5.6.1.1	NR FR1- NR FR1 DL active BWP switch of SCell with non-DRX in SA	2611

A.6.5.6.1.2	NR FR1 DL active BWP switch with non-DRX in SA	2616

A.6.5.6.2	RRC-based Active BWP Switch	2619

A.6.5.6.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	2619

A.6.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	2621

A.6.5.6.3.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	2621

A.6.5.6.4	SCell dormancy switch	2627

A.6.5.6.4.1	NR FR1 PCell SCell dormancy switch of single FR1 SCell outside active time	2627

A.6.5.6.4.1.1	Test Purpose and Environment	2627

A.6.5.6.4.1.2	Test Requirements	2631

A.6.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR1 SCells inside active time	2632

A.6.5.6.4.2.1	 Test Purpose and Environment	2632

A.6.5.6.4.2.2	Test Requirements	2637

A.6.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	2637

A.6.5.6.5.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	2637

A.6.5.7	DL interruptions at switching between two uplink carriers	2641

A.6.5.7.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	2641

A.6.5.7.1.1	Test Purpose and Environment	2641

A.6.5.7.1.2	Test Requirements	2645

A.6.5.7.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	2645

A.6.5.7.2.1	Test Purpose and Environment	2645

A.6.5.7.2.2	Test Requirements	2648

A.6.5.7A	DL interruptions at switching between two uplink carriers with two transmit antenna connectors	2648

A.6.5.7A.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	2648

A.6.5.7A.1.1	Test Purpose and Environment	2648

A.6.5.7A.1.2	Test Requirements	2652

A.6.5.7A.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	2652

A.6.5.7A.2.1	Test Purpose and Environment	2652

A.6.5.7A.2.2	Test Requirements	2654

A.6.5.7B	DL interruptions at switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	2655

A.6.5.7B.1	DL interruptions at switching between two uplink bands in FDD-TDD CA	2655

A.6.5.7B.1.1	Test Purpose and Environment	2655

A.6.5.7B.1.2	Test Requirements	2659

A.6.5.7B.2	DL interruptions at switching between two uplink bands in TDD-TDD CA	2659

A.6.5.7B.2.1	Test Purpose and Environment	2659

A.6.5.7B.2.2	Test Requirements	2664

A.6.5.7C	DL interruptions at switching between two uplink bands with two transmit antenna connectors	2664

A.6.5.7C.1	DL interruptions at switching between two uplink bands with two transmit antenna connectors in FDD-TDD CA	2664

A.6.5.7C.1.1	Test Purpose and Environment	2664

A.6.5.7C.1.2	Test Requirements	2670

A.6.5.7C.2	DL interruptions at switching between two uplink bands with two transmit antenna connectors in TDD-TDD CA	2670

A.6.5.7C.2.1	Test Purpose and Environment	2670

A.6.5.7C.2.2	Test Requirements	2675

A.6.5.7D	DL interruptions at UE switching across three or four uplink bands	2675

A.6.5.7D.1	DL interruptions at switching across three uplink bands in TDD-TDD CA for single TAG	2675

A.6.5.7D.1.1	Test Purpose and Environment	2675

A.6.5.7D.1.2	Test Requirements	2679

A.6.5.7D.2	DL interruptions at switching across four uplink bands in FDD-TDD CA for single TAG	2679

A.6.5.7D.2.1	Test Purpose and Environment	2679

A.6.5.7D.2.2	Test Requirements	2683

A.6.5.7D.3	DL interruptions at switching across three uplink bands in FDD-TDD CA for two TAGs	2683

A.6.5.7D.3.1	Test Purpose and Environment	2683

A.6.5.7D.3.2	Test Requirements	2687

A.6.5.7D.4	DL interruptions at switching across four uplink bands in TDD-TDD CA for two TAGs	2687

A.6.5.7D.4.1	Test Purpose and Environment	2687

A.6.5.7D.7.2	Test Requirements	2692

A.6.5.8	UE specific CBW change	2692

A.6.5.8.1	UE specific CBW change on PCell in FR1 in non-DRX	2692

A.6.5.8.1.1	Test Purpose and Environment	2692

A.6.5.8.1.2	Test Requirements	2694

A.6.5.9	Pathloss reference signal switching delay	2694

A.6.5.9.1	MAC-CE based pathloss reference signal switch delay	2694

A.6.5.9.1.1	Test Purpose and Environment	2694

A.6.5.9.1.2	Test Requirements	2697

A.6.5.10	Conditional PSCell addition and release delay (FR1 NR-DC)	2697

A.6.5.10.1	Conditional PSCell Addition and Release Delay	2697

A.6.5.10.1.1	Test purpose and environment	2697

A.6.5.10.1.2	Test Parameters	2697

A.6.5.10.1.3	Test Requirements	2700

A.6.5.11	PSCell addition and release delay	2700

A.6.5.11.1	Addition and Release Delay of unknown NR FR1 PSCell	2700

A.6.5.11.1.1	Test purpose and environment	2700

A.6.5.11.1.2	Test Requirements	2702

A.6.5.11.2	Addition and Release Delay of unknown NR FR1 PSCell with less than 5 MHz	2703

A.6.5.11.2.1	Test purpose and environment	2703

A.6.5.11.2.2	Test Requirements	2703

A.6.5.12	Subsequent conditional PSCell addition/change	2704

A.6.5.12.1	Intra-frequency subsequent CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC	2704

A.6.5.12.1.1	Test purpose and environment	2704

A.6.5.12.1.2	Test Parameters	2704

A.6.5.12.1.3	Test Requirements	2706

A.6.5.12.2	Inter-frequency subsequent CPA from FR1-FR1 NR-DC to FR1-FR1 NR-DC	2707

A.6.5.12.2.1	Test purpose and environment	2707

A.6.5.12.2.2	Test Parameters	2707

A.6.5.12.2.3	Test Requirements	2710

A.6.5.12.3	Intra-frequency subsequent CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC with 12 PRB SSB bandwidth	2710

A.6.5.12.3.1	Test purpose and environment	2710

A.6.5.12.3.2	Test Parameters	2710

A.6.5.12.3.3	Test Requirements	2711

A.6.5.12.4	Inter-frequency subsequent CPA from FR1-FR1 NR-DC to FR1-FR1 NR-DC with 12 PRB SSB bandwidth	2711

A.6.5.12.4.1	Test purpose and environment	2711

A.6.5.12.4.2	Test Parameters	2711

A.6.5.12.4.3	Test Requirements	2712

A.6.5.13	Active TCI state switch delay	2712

A.6.5.13.1	MAC-CE based joint TCI state switch for mDCI with two TA when RTD is larger than CP	2713

A.6.5.13.1.1	Test Purpose and Environment	2713

A.6.5.13.1.2	Test Requirements	2715

A.7	NR standalone tests with one or more NR cells in FR2	2716

A.7.1	SA: RRC_IDLE state mobility	2716

A.7.1.1	Cell re-selection to NR	2716

A.7.1.1.1	Cell reselection to FR2 intra-frequency NR case	2716

A.7.1.1.1.1	Test Purpose and Environment	2716

A.7.1.1.1.2	Test Parameters	2716

A.7.1.1.1.3	Test Requirements	2718

A.7.1.1.2	Cell reselection to FR2 inter-frequency NR case	2718

A.7.1.1.2.1	Test Purpose and Environment	2719

A.7.1.1.2.2	Test Parameters	2719

A.7.1.1.2.3	Test Requirements	2720

A.7.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	2721

A.7.1.1.3.1	Test Purpose and Environment	2721

A.7.1.1.3.2	Test Parameters	2721

A.7.1.1.3.3	Test Requirements	2723

A.7.1.1.4	Cell reselection to FR2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	2723

A.7.1.1.4.1	Test Purpose and Environment	2723

A.7.1.1.4.2	Test Parameters	2724

A.7.1.1.4.3	Test Requirements	2725

A.7.1.1.5	Cell reselection to FR2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	2726

A.7.1.1.5.1	Test Purpose and Environment	2726

A.7.1.1.5.2	Test Parameters	2726

A.7.1.1.5.3	Test Requirements	2728

A.7.1.1.6	Cell reselection to FR2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	2728

A.7.1.1.6.1	Test Purpose and Environment	2728

A.7.1.1.6.2	Test Parameters	2729

A.7.1.1.6.3	Test Requirements	2730

A.7.1.1.7	Cell reselection to FR2 intra-frequency NR case for FR2 power class 6 UE configured with highSpeedMeasFlagFR2-r17	2731

A.7.1.1.7.1	Test Purpose and Environment	2731

A.7.1.1.7.2	Test Parameters	2731

A.7.1.1.7.3	Test Requirements	2733

A.7.1.1.8	Cell reselection to FR2 inter-frequency NR case for UE configured with highSpeedMeasFlagFR2-r17	2734

A.7.1.1.8.1	Test Purpose and Environment	2734

A.7.1.1.8.2	Test Parameters	2734

A.7.1.1.8.3	Test Requirements	2736

A.7.2	SA: RRC_INACTIVE state mobility	2736

A.7.2.1	Small Data Transmission	2736

A.7.2.1.1	TA validation for CG-SDT in FR2	2736

A.7.2.1.1.1	Test Purpose and Environment	2736

A.7.2.1.1.2	Test Requirements	2739

A.7.2.2	Cell reselection for positioning	2740

A.7.2.2.1	Cell reselection to FR2 intra-frequency NR case with RRC_ INACTIVE eDRX and positioning SRS	2740

A.7.2.2.1.1	Test Purpose and Environment	2740

A.7.2.2.1.2	Test Parameters	2740

A.7.2.2.1.3	Test Requirements	2742

A.7.3	RRC_CONNECTED state mobility	2742

A.7.3.1	Handover	2742

A.7.3.1.1	Inter-frequency handover from FR1 to FR2; unknown target cell	2742

A.7.3.1.1.1	Test Purpose and Environment	2742

A.7.3.1.1.2	Test Parameters	2742

A.7.3.1.1.3	Test Requirements	2744

A.7.3.1.2	Intra-frequency handover from FR2 to FR2; unknown target cell	2745

A.7.3.1.2.1	Test Purpose and Environment	2745

A.7.3.1.2.2	Test Parameters	2745

A.7.3.1.2.3	Test Requirements	2746

A.7.3.1.3	Inter-frequency handover from FR2 to FR2; unknown target cell	2746

A.7.3.1.3.1	Test Purpose and Environment	2746

A.7.3.1.3.2	Test Parameters	2746

A.7.3.1.3.3	Test Requirements	2748

A.7.3.1.4	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR2	2748

A.7.3.1.4.1	Test Purpose and Environment	2748

A.7.3.1.4.2	Test Parameters	2748

A.7.3.1.4.3 Test Requirements	2751

A.7.3.1.5	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR2	2752

A.7.3.1.5.1	Test Purpose and Environment	2752

A.7.3.1.5.2	Test Parameters	2752

A.7.3.1.5.3 Test Requirements	2755

A.7.3.1.6	Handover with PSCell from SA to EN-DC with unknown FR2 target PScell	2756

A.7.3.1.6.1	Test Purpose and Environment	2756

A.7.3.1.6.2	Test Parameters	2756

A.7.3.1.6.3	Test Requirements	2761

A.7.3.1.7	HO with PSCell from FR1 NR-SA to EN-DC with known E-UTRA PCell and known FR2 PSCell	2761

A.7.3.1.7.1	Test purpose and environment	2761

A.7.3.1.7.2	Test Requirements	2765

A.7.3.1.8	NR PSCell change delay in HO with PSCell from NR-DC to NR-DC	2766

A.7.3.1.8.1	Test Purpose and Environment	2766

A.7.3.1.8.2	Test Requirements	2769

A.7.3.1.9	Intra-frequency handover from FR2-2 to FR2-2; unknown target cell	2769

A.7.3.1.9.1	Test Purpose and Environment	2769

A.7.3.1.9.2	Test Parameters	2769

A.7.3.1.9.3	Test Requirements	2771

A.7.3.1.10	Inter-frequency handover from FR2-2 to FR2-2; unknown target cell	2772

A.7.3.1.10.1	Test Purpose and Environment	2772

A.7.3.1.10.2	Test Parameters	2772

A.7.3.1.10.3	Test Requirements	2774

A.7.3.1.11	Inter-frequency handover from FR1 to FR2-2; unknown target cell	2774

A.7.3.1.11.1	Test Purpose and Environment	2774

A.7.3.1.11.2	Test Parameters	2774

A.7.3.1.11.3	Test Requirements	2776

A.7.3.1.12	Intra-frequency handover from FR2 to FR2; known target cell configured with NCD-SSB	2777

A.7.3.1.12.1	Test Purpose and Environment	2777

A.7.3.1.12.2	Test Parameters	2777

A.7.3.1.12.3	Test Requirements	2778

A.7.3.1.13	Inter-frequency handover from FR2 to FR2; known target cell configured with NCD-SSB	2779

A.7.3.1.13.1	Test Purpose and Environment	2779

A.7.3.1.13.2	Test Parameters	2779

A.7.3.1.13.3	Test Requirements	2781

A.7.3.1.14	Handover with PSCell from FR1-FR2 NR-DC to FR1-FR1 NR-DC with target PSCell in FR1	2781

A.7.3.1.14.1	Test Purpose and Environment	2781

A.7.3.1.14.2	Test Requirements	2785

A.7.3.1.15	HO with PSCell from FR1-FR1 NR-DC to FR1-FR2 NR-DC	2785

A.7.3.1.15.1	Test Purpose and Environment	2785

A.7.3.1.15.2	Test Requirements	2790

A.7.3.2	RRC Connection Mobility Control	2790

A.7.3.2.1	SA: RRC Re-establishment	2790

A.7.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	2790

A.7.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	2792

A.7.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	2795

A.7.3.2.1.3.1	Test Purpose and Environment	2795

A.7.3.2.1.3.2	Test Requirements	2796

A.7.3.2.1.4	Intra-frequency RRC Re-establishment in FR2-2	2797

A.7.3.2.1.4.1	Test Purpose and Environment	2797

A.7.3.2.1.4.2	Test Requirements	2799

A.7.3.2.1.5	Inter-frequency RRC Re-establishment in FR2-2	2799

A.7.3.2.1.5.1	Test Purpose and Environment	2799

A.7.3.2.1.5.2	Test Requirements	2801

A.7.3.2.1.6	Intra-frequency RRC Re-establishment in FR2-2 without serving cell timing	2802

A.7.3.2.1.6.1	Test Purpose and Environment	2802

A.7.3.2.1.6.2	Test Requirements	2803

A.7.3.2.2	Random Access	2804

A.7.3.2.2.1	4-step RA type c ontention based random access test in FR2 for NR Standalone	2804

A.7.3.2.2.2	4-step RA type n on-contention based random access test in FR2 for NR Standalone	2807

A.7.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	2810

A.7.3.2.2.4	2-step RA type n on-contention based random access test in FR2 for NR Standalone	2813

A.7.3.2.3	SA: RRC Connection Release with Redirection	2816

A.7.3.2.3.1	Redirection from NR in FR2 to NR in FR2	2816

A.7.3.2.4	LTM PDCCH-order Random Access	2818

A.7.3.2.4.1	PDCCH-order RACH on neighbor cell in FR2 when RACH BW is within active BWP	2818

A.7.3.2.4.2	PDCCH-order RACH on inter-frequency neighbor cell in FR2	2821

A.7.3.3	Conditional Handover	2824

A.7.3.3.1	Intra-frequency conditional handover from FR2 to FR2	2824

A.7.3.3.1.1	Test Purpose and Environment	2824

A.7.3.3.1.2	Test Parameters	2824

A.7.3.3.1.2.3	Test Requirements	2825

A.7.3.3.2	Inter-frequency conditional handover from FR2 to FR2; unknown target cell	2826

A.7.3.3.2.1	Test Purpose and Environment	2826

A.7.3.3.2.2	Test Parameters	2826

A.7.3.3.2.3Test Requirements	2827

A.7.3.3.3	NES triggering intra-frequency target CHO delay From FR2 to FR2	2827

A.7.3.3.3.1	Test Purpose and Environment	2827

A.7.3.3.3.2	Test Parameters	2828

A.7.3.3.3.2.3	Test Requirements	2829

A.7.3.3.4	NES triggering inter-frequency conditional handover from FR2 to FR1	2829

A.7.3.3.4.1	Test Purpose and Environment	2829

A.7.3.3.4.2	Test Parameters	2829

A.7.3.3.4.3	Test Requirements	2831

A.7.3.3.5	NR conditional handover including target MCG and target SCG from FR1-FR2 NR-DC to FR1-FR2 NR-DC	2832

A.7.3.3.5.1	Test Purpose and Environment	2832

A.7.3.3.5.2	Test Requirements	2835

A.7.3.3.5.2.1	Test Requirements for NR conditional handover	2835

A.7.3.3.5.2.2	Test Requirements for NR PSCell change	2835

A.7.3.3.6	NR conditional Handover including target MCG and candidate SCG from FR1-FR2 NR-DC to FR1-FR2 NR-DC	2835

A.7.3.3.6.1	Test Purpose and Environment	2835

A.7.3.3.6.2	Test Parameters	2835

A.7.3.3.6.3 Test Requirements	2839

A.7.3.4	LTM PCell Switch	2839

A.7.3.4.1	RACH based Intra-frequency PCell switch from FR2 to FR2	2839

A.7.3.4.1.1	Test Purpose and Environment	2839

A.7.3.4.1.2	Test Parameters	2839

A.7.3.4.1.3	Test Requirements	2842

A.7.3.4.2	RACH-less Intra-frequency PCell switch from FR2 to FR2	2843

A.7.3.4.2.1	Test Purpose and Environment	2843

A.7.3.4.2.2	Test Parameters	2843

A.7.3.4.2.3	Test Requirements	2847

A.7.3.4.3	RACH-based Inter-frequency LTM PCell switch from FR2 to FR2	2847

A.7.3.4.3.1	Test Purpose and Environment	2847

A.7.3.4.3.2	Test Parameters	2847

A.7.3.4.3.3	Test Requirements	2850

A.7.3.5	LTM PSCell Switch	2851

A.7.3.5.1	RACH-based Intra-frequency LTM PSCell switch from FR2 to FR2	2851

A.7.3.5.1.1	Test Purpose and Environment	2851

A.7.3.5.1.2	Test Parameters	2851

A.7.3.5.1.3	Test Requirements	2855

A.7.4	Timing	2856

A.7.4.1	UE transmit timing	2856

A.7.4.1.1	NR UE Transmit Timing Test for FR2	2856

A.7.4.1.1.1	Test Purpose and environment	2856

A.7.4.1.1.2	Test requirements	2858

A.7.4.1.2	NR UE Transmit Timing Test for FR2-2	2858

A.7.4.1.2.1	Test Purpose and environment	2858

A.7.4.1.2.2	Test requirements	2861

A.7.4.1.3	NR UE Transmit Timing Test with 2-TA for FR2 UE supporting multiDCI-IntraCellMultiTRP-TwoTA-r18	2861

A.7.4.1.3.1	Test Purpose and environment	2861

A.7.4.1.3.2	Test requirements	2865

A.7.4.2	UE timer accuracy	2865

A.7.4.3	Timing advance	2865

A.7.4.3.1	SA FR2 timing advance adjustment accuracy	2865

A.7.4.3.1.1	Test Purpose and Environment	2865

A.7.4.3.1.2	Test Parameters	2866

A.7.4.3.1.3 Test Requirements	2868

A.7.4.3.2	SA FR2-2 timing advance adjustment accuracy	2868

A.7.4.3.2.1	Test Purpose and Environment	2868

A.7.4.3.2.2	Test Parameters	2868

A.7.4.3.2.3	Test Requirements	2870

A.8	E-UTRA standalone tests for NR RRM	2872

A.8.1	Void	2872

A.8.2	RRC_IDLE state mobility	2872

A.8.2.1	Inter-RAT NR Cell re-selection	2872

A.8.2.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	2872

A.8.2.1.1.1	Test Purpose and Environment	2872

A.8.2.1.1.2	Test Requirements	2875

A.8.2.1.2	E-UTRA Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16	2875

A.8.2.1.2.1	Test Purpose and Environment	2875

A.8.2.1.2.2	Test Requirements	2878

A.8.2.2	E-UTRA – NR Inter-RAT Early Measruement Reporting	2879

A.8.2.2.1	E-UTRA – NR Early Measurement Reporting for NR in FR1	2879

A.8.2.2.1.1	Test Purpose and Environment	2879

A.8.2.2.1.2	Test Requirements	2881

A.8.2.2.2	E-UTRA – NR Early Measurement Reporting for NR in FR2	2882

A.8.2.2.2.1	Test Purpose and Environment	2882

A.8.2.2.2.2	Test Requirements	2884

A.8.3	RRC_CONNECTED state mobility	2884

A.8.3.1	Handover	2884

A.8.3.1.1	E-UTRAN - NR handover in FR1	2884

A.8.3.1.1.1	Test Purpose and Environment	2884

A.8.3.1.1.2	Test Requirements	2888

A.8.4	Measurement procedure	2888

A.8.4.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay	2888

A.8.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay in non-DRX	2888

A.8.4.1.1.1	Test Purpose and Environment	2888

A.8.4.1.1.2	Test Requirements	2890

A.8.4.1.2	E-UTRA – NR Inter-RAT SFTD Measurement Delay in DRX	2890

A.8.4.1.2.1	Test Purpose and Environment	2890

A.8.4.1.2.2	Test Requirements	2891

A.8.4.2	E-UTRA – NR Inter-RAT Measurements	2891

A.8.4.2.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2891

A.8.4.2.1.1	Test Purpose and Environment	2891

A.8.4.2.1.2	Test Requirements	2894

A.8.4.2.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2894

A.8.4.2.2.1	Test Purpose and Environment	2894

A.8.4.2.2.2	Test Requirements	2897

A.8.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2898

A.8.4.2.3.1	Test Purpose and Environment	2898

A.8.4.2.3.2	Test Requirements	2901

A.8.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2901

A.8.4.2.4.1	Test Purpose and Environment	2901

A.8.4.2.4.2	Test Requirements	2904

A.8.4.2.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	2904

A.8.4.2.5.1	Test Purpose and Environment	2904

A.8.4.2.5.2	Test Requirements	2906

A.8.4.2.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	2907

A.8.4.2.6.1	Test Purpose and Environment	2907

A.8.4.2.6.2	Test Requirements	2908

A.8.4.2.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	2909

A.8.4.2.7.1	Test Purpose and Environment	2909

A.8.4.2.7.2	Test Requirements	2911

A.8.4.2.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	2911

A.8.4.2.8.1	Test Purpose and Environment	2911

A.8.4.2.8.2	Test Requirements	2913

A.8.4.2.9	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16	2913

A.8.4.2.9.1	Test Purpose and Environment	2913

A.8.4.2.9.2	Test Requirements	2916

A.8.4.3	E-UTRAN - NR Inter-RAT event-triggered without measurement gaps	2917

A.8.4.3.1	NR Inter-RAT event triggered reporting tests for FR2 without MG nor DRX	2917

A.8.4.3.1.1	Test Purpose and Environment	2917

A.8.4.3.1.2	Test Requirements	2918

A.8.4.3.2	NR Inter-RAT event triggered reporting tests for FR1 without gaps when DRX is not used	2919

A.8.4.3.2.1	Test Purpose and Environment	2919

A.8.4.3.2.2	Test Requirements	2922

A.8.5	Measurement performance	2922

A.8.5.1	SFTD accuracy	2922

A.8.5.1.1	SFTD accuracy	2922

A.8.5.1.1.1	Test Purpose	2922

A.8.5.1.1.2	Test Environment	2922

A.8.5.1.1.3	Test Requirements	2926

A.8.5.2	E-UTRA – NR Inter-RAT Measurement Performance requirements	2926

A.8.5.2.1.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	2926

A.8.5.2.1.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	2929

A.8.5.2.1.2.1	Test Purpose and Environment	2929

A.8.5.2.1.2.2	Test Parameters	2929

A.8.5.2.1.2.3	Test Requirements	2931

A.8.5.2.2	SS-RSRQ	2931

A.8.5.2.2.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	2931

A.8.5.2.2.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	2934

A.8.5.2.2.2.1	Test Purpose and Environment	2934

A.8.5.2.2.2.2	Test Parameters	2934

A.8.5.2.2.2.3	Test Requirements	2936

A.8.5.2.3	SS-SINR	2936

A.8.5.2.3.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	2936

A.8.5.2.3.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	2939

A.8.5.2.3.2.1	Test Purpose and Environment	2939

A.8.5.2.3.2.2	Test Parameters	2940

A.8.5.2.3.2.3	Test Requirements	2941

A.9	V2X Tests	2941

A.9.1	V2X Tests in FR1	2941

A.9.1.1	Test for V2X UE Transmit Timing	2941

A.9.1.1.1 Test for GNSS as Synchronization Reference Source	2941

A.9.1.1.1.1	Test Purpose and Environment	2941

A.9.1.1.1.2	Test requirements	2942

A.9.1.1.2	Test for SyncRef UE as Synchronization Reference Source	2942

A.9.1.1.2.1	Test Purpose and Environment	2942

A.9.1.1.2.2	Test requirements	2942

A.9.1.1.3	Test for FR1 NR Cell as Synchronization Reference Source	2942

A.9.1.1.3.1	Test Purpose and Environment	2943

A.9.1.1.3.2	Test requirements	2944

A.9.1.2	Test for Initiation/Cease of S-SSB Transmission with V2X Sidelink Communication	2944

A.9.1.2.1	Test for FR1 NR Cell as synchronization reference source without gap under non-DRX	2944

A.9.1.2.1.1	Test Purpose and Environment	2944

A.9.1.2.1.2	Test Requirements	2946

A.9.1.2.2	Test for SyncRef UE as synchronization reference source	2946

A.9.1.2.2.1	Test Purpose and Environment	2946

A.9.1.2.2.2	Test Requirements	2948

A.9.1.2.3	Test for SyncRef UE as synchronization reference source when SL-DRX is used	2948

A.9.1.2.3.1	Test Purpose and Environment	2948

A.9.1.2.3.2	Test Requirements	2950

A.9.1.2.4	Test for SyncRef UE as synchronization reference source with CCA	2950

A.9.1.2.4.1	Test Purpose and Environment	2950

A.9.1.2.4.2	Test Requirements	2951

A.9.1.3	 Test for V2X Synchronization Reference Selection/Reselection	2952

A.9.1.3.1	 Test for GNSS configured as the highest priority	2952

A.9.1.3.1.1	Test Purpose and Environment	2952

A.9.1.3.1.2	Test Requirements	2953

A.9.1.3.2	 Test for FR1 NR Cell configured as the highest priority	2954

A.9.1.3.2.1	Test Purpose and Environment	2954

A.9.1.3.2.2	Test Requirements	2956

A.9.1.3.3	Test for GNSS configured as the highest priority under SL-DRX	2957

A.9.1.3.3.1	Test Purpose and Environment	2957

A.9.1.3.3.2	Test Requirements	2958

A.9.1.3.4	Test for FR1 NR Cell configured as the highest priority under SL-DRX	2959

A.9.1.3.4.1	Test Purpose and Environment	2959

A.9.1.3.4.2	Test Requirements	2961

A.9.1.4	Test for L1 SL-RSRP Measurement	2961

A.9.1.4.1	Test for V2X UE Autonomous Resource Selection/Reselection	2961

A.9.1.4.1.1	Test Purpose and Environment	2961

A.9.1.4.1.2	Test Requirements	2963

A.9.1.4.2	Test for V2X UE Resource Pre-emption	2964

A.9.1.4.2.1	Test Purpose and Environment	2964

A.9.1.4.2.2	Test Requirements	2965

A.9.1.4.3	 Test for V2X UE Resource Re-evaluation	2966

A.9.1.4.3.1	Test Purpose and Environment	2966

A.9.1.4.3.2	Test Requirements	2969

A.9.1.4.4	Test for V2X UE Autonomous Resource Selection/Reselection with Periodic Sensing	2969

A.9.1.4.4.1	Test Purpose and Environment	2969

A.9.1.4.4.2	Test Requirements	2971

A.9.1.4.5	Test for V2X UE Autonomous Resource Selection/Reselection with Contiguous Sensing	2971

A.9.1.4.5.1	Test Purpose and Environment	2971

A.9.1.4.5.2	Test Requirements	2973

A.9.1.4.6	Test for V2X UE Autonomous Resource Selection/Reselection in SL-DRX	2974

A.9.1.4.6.1	Test Purpose and Environment	2974

A.9.1.4.6.2	Test Requirements	2976

A.9.1.5	Test for Congestion Control Measurement	2976

A.9.1.5.1	Test Purpose and Environment	2976

A.9.1.5.2	Test Requirements	2979

A.9.1.6	Test for Interruption	2979

A.9.1.6.1	Test for Interruption to WAN due to V2X Sidelink Communication	2979

A.9.1.6.1.1	Test Purpose and Environment	2979

A.9.1.6.1.2	Test Requirements	2982

A.9.1.6.2	Test for interruption to WAN at transitions between active and non-active during SL-DRX in asynchronous case	2982

A.9.1.6.2.1	Test Purpose and Environment	2982

A.9.1.6.2.2	Test Requirements	2984

A.9.1.6.3	Test for Interruption at NR Sidelink Diccovery Configuration	2984

A.9.1.6.3.1	Test Purpose and Environment	2984

A.9.1.6.3.2	Test Requirements	2987

A.9.1.7	Selection / Reselection of relay UE	2987

A.9.1.7.1	Test Purpose and Environment	2987

A.9.1.7.2	Test Requirements	2991

A.9A	Tests for NR Sidelink Measurements for Positioning	2992

A.9A.1	Tests for NR Sidelink Measurements for Positioning in FR1	2992

A.9A.1.1	Measurement delay tests	2992

A.9A.1.1.1	NR SL RSTD measurement reporting delay test case in FR1 SA	2992

A.9A.1.1.1.1	Test Purpose and Environment	2992

A.9A.1.1.1.2	Test Requirements	2997

A.9A.1.1.2	SL Rx-Tx measurement delay tests	2997

A.9A.1.1.2.1	Test Purpose and Environment	2997

A.9A.1.1.2.2	Test Requirements	3000

A.9A.1.1.3	NR SL AoA measurements reporting delay test in FR1 SA	3001

A.9A.1.1.3.1	Test Purpose and Environment	3001

A.9A.1.1.3.2	Test Requirements	3004

A.9A.1.1.4	NR SL RTOA measurements reporting delay test in FR1 SA	3005

A.9A.1.1.4.1	Test Purpose and Environment	3005

A.9A.1.1.4.2	Test Requirements	3008

A.9A.1.1.5	NR SL PRS-RSRP measurement reporting delay test case in FR1 SA	3009

A.9A.1.1.5.1	Test Purpose and Environment	3009

A.9A.1.1.5.2	Test Requirements	3009

A.9A.1.1.6	NR SL PRS-RSRPP measurement reporting delay test case in FR1 SA	3009

A.9A.1.1.6.1	Test Purpose and Environment	3009

A.9A.1.1.6.2	Test Requirements	3009

A.9A.1.2	Measurement Accuracy Tests	3010

A.9A.1.2.1	NR SL RSTD measurement accuracy test case in FR1 SA	3010

A.9A.1.2.1.1	Test Purpose and Environment	3010

A.9A.1.2.1.2	Test Requirements	3013

A.9A.1.2.2	SL Rx-Tx measurement accuracy test case in FR1	3014

A.9A.1.2.2.1	Test Purpose and Environment	3014

A.9A.1.2.2.2	Test Requirements	3017

A.9A.1.2.3	NR SL PRS-RSRP measurement accuracy test case in FR1 SA	3017

A.9A.1.2.3.1	Test Purpose and Environment	3017

A.9A.1.2.3.2	Test Requirements	3018

A.9A.1.2.4	NR SL PRS-RSRPP measurement accuracy test case in FR1 SA	3018

A.9A.1.2.4.1	Test Purpose and Environment	3018

A.9A.1.2.4.2	Test Requirements	3018

A.10	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	3020

A.10.1	RRC_CONNECTED state mobility	3020

A.10.1.1	RRC connection mobility control	3020

A.10.1.1.1	Random Access	3020

A.10.1.1.1.1	4-step RA type contention-based random access for NR PSCell with CCA	3020

A.10.1.1.1.1.1	Test Purpose and Environment	3020

A.10.1.1.1.1.2	Test Requirements	3021

A.10.1.1.1.1.2.1	Random Access Preamble Transmission	3022

A.10.1.1.1.1.2.2	Random Access Response Reception	3022

A.10.1.1.1.1.2.3	No Random Access Response Reception	3022

A.10.1.1.1.1.2.4	Receiving an UL grant for msg3 retransmission	3023

A.10.1.1.1.1.2.5	 Contention Resolution Timer expiry	3023

A.10.1.1.1.2	4-step RA type non-contention based random access for NR PSCell with CCA	3023

A.10.1.1.1.2.1	Test Purpose and Environment	3023

A.10.1.1.1.2.2	Test Requirements	3024

A.10.1.1.1.2.2.1	SSB-based Random Access Preamble Transmission	3025

A.10.1.1.1.2.2.2	Random Access Response Reception	3025

A.10.1.1.1.2.2.3	No Random Access Response Reception	3025

A.10.1.1.1.3	2-step RA type contention-based random access for NR PSCell with CCA	3026

A.10.1.1.1.3.1	Test Purpose and Environment	3026

A.10.1.1.1.3.2	Test Requirements	3027

A.10.1.1.1.3.2.1	MsgA Transmission	3028

A.10.1.1.1.3.2.2	MsgB Reception	3028

A.10.1.1.1.3.2.3	No MsgB Reception	3028

A.10.1.1.1.4	2-step RA type non-contention based random access for NR PSCell with CCA	3029

A.10.1.1.1.4.1	Test Purpose and Environment	3029

A.10.1.1.1.4.2	Test Requirements	3030

A.10.1.1.1.4.2.1	MsgA Transmission	3031

A.10.1.1.1.4.2.2	MsgB Reception	3031

A.10.1.1.1.4.2.3	No MsgB Reception	3031

A.10.1.2	Handover with PSCell from EN-DC to EN-DC with known target PSCell using CCA	3032

A.10.1.2.1	Test Purpose and Environment	3032

A.10.1.2.2	Test Requirements	3036

A.10.2	Timing	3037

A.10.2.1	UE transmit timing	3037

A.10.2.1.1	UE Transmit Timing Test with PSCell under DL CCA	3037

A.10.2.1.1.1	Test Purpose and environment	3037

A.10.2.1.1.2	Test requirements	3039

A.10.2.2	UE timing advance	3040

A.10.2.2.1	UE Timing Advance Adjustment Accuracy with PSCell under DL CCA	3040

A.10.2.2.1.1	Test Purpose and Environment	3040

A.10.2.2.1.2	Test Parameters	3040

A.10.2.2.1.3	Test Requirements	3042

A.10.3	Signalling characteristics	3042

A.10.3.1	Radio link monitoring	3042

A.10.3.1.1	Introduction	3042

A.10.3.1.2	Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	3043

A.10.3.1.2.1	Test purpose and environment	3043

A.10.3.1.2.2	Test requirements	3045

A.10.3.1.3	Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	3046

A.10.3.1.3.1	Test purpose and environment	3046

A.10.3.1.3.2	Test requirements	3048

A.10.3.1.4	Void	3048

A.10.3.1.4.1	Void	3048

A.10.3.1.4.2	Void	3048

A.10.3.1.5	Void	3049

A.10.3.1.5.1	Void	3049

A.10.3.1.5.2	Void	3049

A.10.3.2	Void	3049

A.10.3.3	SCell activation and deactivation delay	3049

A.10.3.3.1	SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 160 ms SCell measurement cycle	3049

A.10.3.3.1.1	Test Purpose and Environment	3049

A.10.3.3.1.2	Test Requirements	3052

A.10.3.3.2 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 640 ms SCell measurement cycle	3052

A.10.3.3.2.1	Test Purpose and Environment	3052

A.10.3.3.2.2	Test Requirements	3053

A.10.3.3.3	SCell Activation and Deactivation of unknown NR SCell with NR PSCell and NR SCell under CCA	3053

A.10.3.3.3.1	Test Purpose and Environment	3053

A.10.3.3.3.2	Test Requirements	3053

A.10.3.4	Beam failure detection and link recovery procedures	3054

A.10.3.4.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	3054

A.10.3.4.1.1	Test Purpose and Environment	3054

A.10.3.4.1.2	Test Requirements	3057

A.10.3.4.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	3057

A.10.3.4.2.1	Test Purpose and Environment	3057

A.10.3.4.2.2	Test Requirements	3060

A.10.3.5	Active BWP switching	3061

A.10.3.5.1	UL active BWP switch delay with consistent UL LBT failure on PSCell subject to UL CCA in EN-DC	3061

A.10.3.5.1.2	Test Requirements	3063

A.10.3.5.2	DCI-based and Timer-based Active BWP Switch	3064

A.10.3.5.2.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	3064

A.10.3.5.2.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	3067

A.10.3.5.3	RRC-based Active BWP Switch	3071

A.10.3.5.3.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	3071

A.10.3.6	PSCell addition and release delay	3073

A.10.3.6.1	Addition and Release Delay of known NR PSCell on the carrier under CCA	3073

A.10.3.6.1.1	Test purpose and environment	3073

A.10.3.6.1.2	Test Requirements	3076

A.10.3.7	Void	3076

A.10.4	Measurement procedure	3076

A.10.4.1	Intra-frequency measurements	3077

A.10.4.1.1	Event-triggered reporting tests on PSCC without gaps under non-DRX	3077

A.10.4.1.1.1	Test purpose and environment	3077

A.10.4.1.1.2	Test parameters	3077

A.10.4.1.1.3	Test Requirements	3079

A.10.4.1.2	Void	3079

A.10.4.1.3	Void	3079

A.10.4.1.4	Event-triggered reporting tests on PSCC with per-UE gaps under DRX	3079

A.10.4.1.4.1	Test purpose and environment	3079

A.10.4.1.4.2	Test parameters	3079

A.10.4.1.4.3	Test Requirements	3082

A.10.4.1.5	Void	3082

A.10.4.1.6	Void	3082

A.10.4.1.7	Void	3082

A.10.4.1.8	Void	3082

A.10.4.1.9	Void	3082

A.10.4.1.10	Void	3082

A.10.4.1.11	Void	3082

A.10.4.1.12	Void	3082

A.10.4.2	Inter-frequency measurements	3082

A.10.4.2.1	Void	3082

A.10.4.2.2	Void	3082

A.10.4.2.3	EN-DC event triggered reporting tests for FR1 with CCA cell without SSB time index detection when DRX is not used	3083

A.10.4.2.3.1	Test Purpose and Environment	3083

A.10.4.2.3.2	Test Requirements	3085

A.10.4.2.4	EN-DC event triggered reporting tests for FR1 cell with CCA without SSB time index detection when DRX is used	3085

A.10.4.2.4.1	Test Purpose and Environment	3085

A.10.4.2.4.2	Test Requirements	3088

A.10.4.2.5	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is not used	3089

A.10.4.2.5.1	Test Purpose and Environment	3089

A.10.4.2.5.2	Test Requirements	3091

A.10.4.2.6	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is used	3091

A.10.4.2.6.1	Test Purpose and Environment	3091

A.10.4.2.6.2	Test Requirements	3094

A.10.4.2.7	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	3095

A.10.4.2.7.1	Test Purpose and Environment	3095

A.10.4.2.7.2	Test Requirements	3098

A.10.4.2.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	3098

A.10.4.2.8.1	Test Purpose and Environment	3098

A.10.4.2.8.2	Test Requirements	3102

A.10.4.2.9	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	3103

A.10.4.2.9.1	Test Purpose and Environment	3103

A.10.4.2.9.2	Test Requirements	3106

A.10.4.2.10	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	3106

A.10.4.2.10.1	Test Purpose and Environment	3106

A.10.4.2.10.2	Test Requirements	3110

A.10.4.3	L1-RSRP measurements for beam reporting	3111

A.10.4.3.1	SSB based L1-RSRP measurement on PSCC when DRX is not used	3111

A.10.4.3.1.1	Test Purpose and Environment	3111

A.10.4.3.1.2	Test parameters	3111

A.10.4.3.1.3	Test Requirements	3112

A.10.4.3.2	SSB based L1-RSRP measurement on PSCC when DRX is used	3113

A.10.4.3.2.1	Test Purpose and Environment	3113

A.10.4.3.2.2	Test parameters	3113

A.10.4.3.2.3	Test Requirements	3114

A.10.4.3.3	SSB based L1-RSRP measurement on SCC when DRX is not used	3115

A.10.4.3.3.1	Test Purpose and Environment	3115

A.10.4.3.3.2	Test parameters	3115

A.10.4.3.3.3	Test Requirements	3116

A.10.4.3.4	SSB based L1-RSRP measurement on SCC when DRX is used	3117

A.10.4.3.4.1	Test Purpose and Environment	3117

A.10.4.3.4.2	Test parameters	3117

A.10.4.3.4.3	Test Requirements	3118

A.10.4.4	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	3119

A.10.4.4.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	3119

A.10.4.4.1.1	Test Purpose and Environment	3119

A.10.4.4.1.2	Test Requirements	3122

A.10.4.4.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	3123

A.10.4.4.2.1	Test Purpose and Environment	3123

A.10.4.4.2.2	Test Requirements	3126

A.10.4.4.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	3127

A.10.4.4.3.1	Test Purpose and Environment	3127

A.10.4.4.3.2	Test Requirements	3130

A.10.4.4.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	3130

A.10.4.4.4.1	Test Purpose and Environment	3130

A.10.4.4.4.2	Test Requirements	3134

A.10.5	Measurement performance	3134

A.10.5.1	SS-RSRP	3134

A.10.5.1.1	Intra-frequency measurement accuracy on a CCA serving cell	3134

A.10.5.1.1.1	Test Purpose and Environment	3135

A.10.5.1.1.2	Test parameters	3135

A.10.5.1.1.3	Test Requirements	3136

A.10.5.1.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	3136

A.10.5.1.2.1	Test Purpose and Environment	3136

A.10.5.1.2.2	Test parameters	3137

A.10.5.1.2.3	Test Requirements	3138

A.10.5.2	SS-RSRQ	3138

A.10.5.2.1	Intra-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	3138

A.10.5.2.1.1	Test Purpose and Environment	3138

A.10.5.2.1.2	Test Parameters	3138

A.10.5.2.1.3	Test Requirements	3140

A.10.5.2.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	3140

A.10.5.2.2.1	Test Purpose and Environment	3140

A.10.5.2.2.2	Test Parameters	3140

A.10.5.2.2.3	Test Requirements	3142

A.10.5.3	SS-SINR	3142

A.10.5.3.1	Intra-frequency measurement accuracy on PSCC	3142

A.10.5.3.1.1	Test Purpose and Environment	3142

A.10.5.3.1.2	Test Parameters	3142

A.10.5.3.1.3	Test Requirements	3143

A.10.5.3.2	Inter-frequency measurement accuracy on PSCC	3144

A.10.5.3.2.1	Test Purpose and Environment	3144

A.10.5.3.2.2	Test Parameters	3144

A.10.5.3.2.3	Test Requirements	3145

A.10.5.3.3	Intra-frequency measurement accuracy on SCC	3145

A.10.5.3.3.1	Test Purpose and Environment	3145

A.10.5.3.3.2	Test Parameters	3145

A.10.5.3.3.3	Test Requirements	3147

A.10.5.4	L1-RSRP measurement for beam reporting with CCA serving cell	3147

A.10.5.4.1	SSB based L1-RSRP measurement	3147

A.10.5.4.1.1	Test Purpose and Environment	3147

A.10.5.4.1.2	Test parameters	3148

A.10.5.4.1.3	Test Requirements	3149

A.10.5.5	RSSI	3149

A.10.5.5.1 	RSSI measurement accuracy on PSCC with CCA	3149

A.10.5.5.1.1	Test Purpose and Environment	3149

A.10.5.5.1.2	Test parameters	3149

A.10.5.5.1.3	Test Requirements	3151

A.10.5.5.2	RSSI measurement accuracy on SCC with CCA	3151

A.10.5.5.2.1	Test Purpose and Environment	3151

A.10.5.5.2.2	Test parameters	3151

A.10.5.5.2.3	Test Requirements	3152

A.10.5.5.3 	Inter-frequency RSSI measurement accuracy on a carrier with CCA	3152

A.10.5.5.3.1	Test Purpose and Environment	3152

A.10.5.5.3.2	Test parameters	3153

A.10.5.5.3.3	Test Requirements	3154

A.10.5.6	Channel occupancy	3154

A.10.5.6.1 	Channel occupancy measurement accuracy on PSCC with CCA	3154

A.10.5.6.1.1	Test Purpose and Environment	3154

A.10.5.6.1.2	Test parameters	3154

A.10.5.6.1.3	Test Requirements	3156

A.10.5.6.2 	Channel occupancy measurement accuracy on SCC with CCA	3156

A.10.5.6.2.1	Test Purpose and Environment	3156

A.10.5.6.2.2	Test parameters	3156

A.10.5.6.2.3	Test Requirements	3158

A.10.5.6.3 	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	3158

A.10.5.6.3.1	Test Purpose and Environment	3158

A.10.5.6.3.2	Test parameters	3158

A.10.5.6.3.3	Test Requirements	3160

A.11	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	3162

A.11.1	RRC_IDLE state mobility	3162

A.11.1.1	Cell re-selection with both source and target NR carrier frequencies under CCA	3162

A.11.1.1.1	Cell reselection to FR1 intra-frequency NR cells when subject to CCA on the serving and target cell	3162

A.11.1.1.1.1	Test Purpose and Environment	3162

A.11.1.1.1.2	Test Parameters	3162

A.11.1.1.1.3	Test Requirements	3164

A.11.1.1.2	Cell reselection to FR1 inter-frequency NR case when subject to CCA on the serving and target cell	3165

A.11.1.1.2.1	Test Purpose and Environment	3165

A.11.1.1.2.2	Test Parameters	3165

A.11.1.1.2.3	Test Requirements	3167

A.11.1.2	Cell re-selection to NR with source NR carrier frequency under CCA	3167

A.11.1.2.1	Cell reselection to FR1 inter-frequency NR case when serving cell is subject to CCA	3167

A.11.1.2.1.1	Test Purpose and Environment	3167

A.11.1.2.1.2	Test Parameters	3168

A.11.1.2.1.3	Test Requirements	3170

A.11.1.3	Cell re-selection from NR carrier with target NR carrier frequency under CCA	3171

A.11.1.3.1	Cell reselection to FR1 inter-frequency NR case when target cell is subject to CCA	3171

A.11.1.3.1.1	Test Purpose and Environment	3171

A.11.1.3.1.2	Test Parameters	3171

A.11.1.3.1.3	Test Requirements	3174

A.11.1.4	Inter-RAT cell re-selection to E-UTRAN with source NR carrier frequency under CCA	3175

A.11.1.4.1	Cell reselection to higher priority E-UTRAN when serving cell is subject to CCA	3175

A.11.1.4.1.1	Test Purpose and Environment	3175

A.11.1.4.1.2	Test Parameters	3175

A.11.1.4.1.3	Test Requirements	3178

A.11.1.4.2	Cell reselection to lower priority E-UTRAN when serving cell is subject to CCA	3178

A.11.1.4.2.1	Test Purpose and Environment	3178

A.11.1.4.2.2	Test Requirements	3180

A.11.2	RRC_CONNECTED state mobility	3181

A.11.2.1	Handover	3181

A.11.2.1.1	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; known target cell	3181

A.11.2.1.1.1	Test Purpose and Environment	3181

A.11.2.1.1.2	Test Parameters	3181

A.11.2.1.1.3 Test Requirements	3183

A.11.2.1.2	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	3184

A.11.2.1.2.1	Test Purpose and Environment	3184

A.11.2.1.2.2	Test Parameters	3184

A.11.2.1.2.3	Test Requirements	3186

A.11.2.1.3	Inter-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	3186

A.11.2.1.3.1	Test Purpose and Environment	3186

A.11.2.1.3.2	Test Parameters	3186

A.11.2.1.3.3	Test Requirements	3188

A.11.2.1.4	Inter-frequency handover from FR1 carrier under CCA to FR1; known target cell	3189

A.11.2.1.4.1	Test Purpose and Environment	3189

A.11.2.1.4.2	Test Parameters	3189

A.11.2.1.4.3	Test Requirements	3192

A.11.2.1.5	Inter-frequency handover from FR1 carrier under CCA to FR1; unknown target cell	3192

A.11.2.1.5.1	Test Purpose and Environment	3192

A.11.2.1.5.2	Test Parameters	3192

A.11.2.1.5.3 Test Requirements	3195

A.11.2.1.6	Inter-frequency handover from FR1 to FR1 carrier under CCA; unknown target cell	3195

A.11.2.1.6.1	Test Purpose and Environment	3195

A.11.2.1.6.2	Test Parameters	3195

A.11.2.1.6.3	Test Requirements	3198

A.11.2.1.7	 SA NR FR1 carrier under CCA - E-UTRAN handover with known target cell	3198

A.11.2.1.7.1	Test Purpose and Environment	3198

A.11.2.1.7.2	Test Requirements	3201

A.11.2.1.8	SA NR FR1 carrier under CCA - E-UTRAN handover with unknown target cell	3202

A.11.2.1.8.1	Test Purpose and Environment	3202

A.11.2.1.8.2	Test Requirements	3205

A.11.2.1.9	Handover with PSCell from NR SA to EN-DC with known target PSCell using CCA	3205

A.11.2.1.9.1	Test Purpose and Environment	3205

A.11.2.1.9.2	Test Requirements	3211

A.11.2.2	RRC connection mobility control	3212

A.11.2.2.1	RRC re-establishment	3212

A.11.2.2.1.1	Intra-frequency RRC Re-establishment with CCA in FR1	3212

A.11.2.2.1.2	Inter-frequency RRC Re-establishment with CCA in FR1	3215

A.11.2.2.1.4	Inter-frequency RRC Re-establishment from NR FR1 carrier without CCA to NR FR1 carrier under CCA	3221

A.11.2.2.2	Random Access	3224

A.11.2.2.2.1	4-step RA type contention-based random access for NR PCell with CCA	3224

A.11.2.2.2.1.1	Test Purpose and Environment	3224

A.11.2.2.2.1.2	Test Requirements	3225

A.11.2.2.2.1.2.1	Random Access Preamble Transmission	3225

A.11.2.2.2.1.2.2	Random Access Response Reception	3226

A.11.2.2.2.1.2.3	No Random Access Response Reception	3226

A.11.2.2.2.1.2.4	Receiving an UL grant for msg3 retransmission	3226

A.11.2.2.2.1.2.5	Reception of an Incorrect Message over Temporary C-RNTI	3226

A.11.2.2.2.1.2.6	Reception of a Correct Message over Temporary C-RNTI	3227

A.11.2.2.2.1.2.7	Contention Resolution Timer expiry	3227

A.11.2.2.2.2	4-step RA type non-contention based random access for NR PSCell with CCA	3227

A.11.2.2.2.2.1	Test Purpose and Environment	3227

A.11.2.2.2.2.2	Test Requirements	3228

A.11.2.2.2.2.2.1	SSB-based Random Access Preamble Transmission	3229

A.11.2.2.2.2.2.2	Random Access Response Reception	3229

A.11.2.2.2.2.2.3	No Random Access Response Reception	3229

A.11.2.2.2.3	2-step RA type contention-based random access for NR PCell with CCA	3230

A.11.2.2.2.3.1	Test Purpose and Environment	3230

A.11.2.2.2.3.2	Test Requirements	3231

A.11.2.2.2.3.2.1	MsgA Transmission	3231

A.11.2.2.2.3.2.2	MsgB Reception	3232

A.11.2.2.2.3.2.3	No MsgB Reception	3232

A.11.2.2.2.4	2-step RA type non-contention-based random access for NR PCell with CCA	3233

A.11.2.2.2.4.1	Test Purpose and Environment	3233

A.11.2.2.2.4.2	Test Requirements	3234

A.11.2.2.2.4.2.1	MsgA Transmission	3234

A.11.2.2.2.4.2.2	MsgB Reception	3235

A.11.2.2.2.4.2.3	No MsgB Reception	3235

A.11.2.2.3	RRC connection release with redirection	3236

A.11.2.2.3.1	Redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA	3236

A.11.2.2.3.2	Redirection from NR FR1 carrier without CCA to NR FR1 carrier with CCA	3238

A.11.3	Timing	3241

A.11.3.1	UE transmit timing	3241

A.11.3.1.1	UE Transmit Timing Test with PCell under DL CCA	3241

A.11.3.1.1.1	Test Purpose and environment	3241

A.11.3.1.1.2	Test requirements	3243

A.11.3.2	UE timing advance	3244

A.11.3.2.1	UE Timing Advance Adjustment Accuracy with PCell under DL CCA	3244

A.11.3.2.1.1	Test Purpose and Environment	3244

A.11.3.2.1.2	Test Parameters	3244

A.11.3.2.1.3	Test Requirements	3246

A.11.4	Signalling characteristics	3246

A.11.4.1	Radio link monitoring	3246

A.11.4.1.1	Introduction	3246

A.11.4.1.2	Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	3247

A.11.4.1.2.1	Test purpose and environment	3247

A.11.4.1.2.2	Test requirements	3249

A.11.4.1.3	Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	3250

A.11.4.1.3.1	Test purpose and environment	3250

A.11.4.1.3.2	Test requirements	3253

A.11.4.1.4	Void	3253

A.11.4.1.4.1	Void	3253

A.11.4.1.4.2	Void	3253

A.11.4.1.5	Void	3253

A.11.4.1.5.1	Void	3253

A.11.4.1.5.2	Void	3253

A.11.4.2	Void	3253

A.11.4.3	SCell activation and deactivation delay	3253

A.11.4.3.1	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 160 ms SCell measurement cycle	3253

A.11.4.3.1.1	Test Purpose and Environment	3253

A.11.4.3.1.2	Test Requirements	3256

A.11.4.3.2	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 640 ms SCell measurement cycle	3256

A.11.4.3.2.1	Test Purpose and Environment	3256

A.11.4.3.2.2	Test Requirements	3257

A.11.4.3.3	SCell Activation and Deactivation of unknown SCell with PCell and SCell under CCA	3257

A.11.4.3.3.1	Test Purpose and Environment	3257

A.11.4.3.3.2	Test Requirements	3257

A.11.4.4	Beam failure detection and link recovery procedures	3258

A.11.4.4.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	3258

A.11.4.4.1.1	Test Purpose and Environment	3258

A.11.4.4.1.2	Test Requirements	3261

A.11.4.4.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	3262

A.11.4.4.2.1	Test Purpose and Environment	3262

A.11.4.4.2.2	Test Requirements	3265

A.11.4.5	Active BWP switching	3265

A.11.4.5.1	UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA	3265

A.11.4.5.1.1	Test Purpose and Environment	3265

A.11.4.5.1.2	Test Requirements	3268

A.11.4.5.2	DCI-based and Timer-based Active BWP Switch	3268

A.11.4.5.2.1	NR FR1- NR FR1 DL active BWP switch of PCell with non-DRX in SA	3268

A.11.4.5.2.2	NR FR1 DL active BWP switch with non-DRX in SA	3271

A.11.4.5.3	RRC-based Active BWP Switch	3274

A.11.4.5.3.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	3274

A.11.4.6	Void	3276

A.11.5	Measurement procedure	3276

A.11.5.1	Intra-frequency measurements	3276

A.11.5.1.1	Event-triggered reporting tests on PCC without gaps under non-DRX	3276

A.11.5.1.1.1	Test purpose and environment	3276

A.11.5.1.1.2	Test parameters	3276

A.11.5.1.1.3	Test Requirements	3278

A.11.5.1.2	Event-triggered reporting tests on PCC without gaps under DRX	3278

A.11.5.1.2.1	Test purpose and environment	3278

A.11.5.1.2.2	Test parameters	3278

A.11.5.1.2.3	Test Requirements	3280

A.11.5.1.3	Void	3281

A.11.5.1.4	Void	3281

A.11.5.1.5	Void	3281

A.11.5.1.6	Void	3281

A.11.5.1.7	Void	3281

A.11.5.1.8	Void	3281

A.11.5.1.9	Void	3281

A.11.5.1.10	Void	3281

A.11.5.1.11	Void	3281

A.11.5.1.12	Void	3281

A.11.5.2	Inter-frequency measurements	3281

A.11.5.2.1	Void	3281

A.11.5.2.2	Void	3281

A.11.5.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	3281

A.11.5.2.3.1	Test Purpose and Environment	3281

A.11.5.2.3.2	Test Requirements	3284

A.11.5.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	3284

A.11.5.2.4.1	Test Purpose and Environment	3284

A.11.5.2.4.2	Test Requirements	3287

A.11.5.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	3287

A.11.5.2.5.1	Test Purpose and Environment	3287

A.11.5.2.5.2	Test Requirements	3290

A.11.5.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	3290

A.11.5.2.6.1	Test Purpose and Environment	3290

A.11.5.2.6.2	Test Requirements	3293

A.11.5.2.7	Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	3294

A.11.5.2.7.1	Test Purpose and Environment	3294

A.11.5.2.7.2	Test Requirements	3296

A.11.5.2.8	Event triggered reporting tests for FR1 without SSB time index detection when DRX is used	3297

A.11.5.2.8.1	Test Purpose and Environment	3297

A.11.5.2.8.2	Test Requirements	3300

A.11.5.2.9	Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	3300

A.11.5.2.9.1	Test Purpose and Environment	3300

A.11.5.2.9.2	Test Requirements	3303

A.11.5.2.10	Event triggered reporting tests for FR1 with SSB time index detection when DRX is used	3303

A.11.5.2.10.1	Test Purpose and Environment	3303

A.11.5.2.10.2	Test Requirements	3307

A.11.5.3	Inter-RAT E-UTRAN measurements	3307

A.11.5.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	3307

A.11.5.3.1.1	Test Purpose and Environment	3307

A.11.5.3.1.2	Test Requirements	3310

A.11.5.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	3310

A.11.5.3.2.1	Test Purpose and Environment	3310

A.11.5.3.2.2	Test Requirements	3314

A.11.5.4	L1-RSRP measurements for beam reporting	3314

A.11.5.4.1	SSB based L1-RSRP measurement when DRX is not used	3314

A.11.5.4.1.1	Test Purpose and Environment	3314

A.11.5.4.1.2	Test parameters	3314

A.11.5.4.1.3	Test Requirements	3316

A.11.5.4.2	SSB based L1-RSRP measurement when DRX is used	3316

A.11.5.4.2.1	Test Purpose and Environment	3316

A.11.5.4.2.2	Test parameters	3316

A.11.5.4.2.3	Test Requirements	3318

A.11.5.4.3	SSB based L1-RSRP measurement on SCC when DRX is not used	3318

A.11.5.4.3.1	Test Purpose and Environment	3318

A.11.5.4.3.2	Test parameters	3319

A.11.5.4.3.3	Test Requirements	3320

A.11.5.4.4	SSB based L1-RSRP measurement on SCC when DRX is used	3320

A.11.5.4.4.1	Test Purpose and Environment	3320

A.11.5.4.4.2	Test parameters	3321

A.11.5.4.4.3	Test Requirements	3322

A.11.6	Measurement performance	3322

A.11.6.1	SS-RSRP	3322

A.11.6.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	3322

A.11.6.1.1.1	Test Purpose and Environment	3322

A.11.6.1.1.2	Test parameters	3323

A.11.6.1.1.3	Test Requirements	3324

A.11.6.1.2	Intra-frequency measurement accuracy on SCC on a carrier frequency with CCA	3324

A.11.6.1.2.1	Test Purpose and Environment	3324

A.11.6.1.2.2	Test parameters	3324

A.11.6.1.2.3	Test Requirements	3326

A.11.6.2	SS-RSRQ	3326

A.11.6.2.1	Intra-frequency measurement accuracy	3326

A.11.6.2.1.1	Test Purpose and Environment	3326

A.11.6.2.1.2	Test Parameters	3326

A.11.6.2.1.3	Test Requirements	3328

A.11.6.2.2	Inter-frequency measurement accuracy	3328

A.11.6.2.2.1	Test Purpose and Environment	3328

A.11.6.2.2.2	Test Parameters	3328

A.11.6.2.2.3	Test Requirements	3331

A.11.6.2.3	Intra-frequency measurement accuracy on SCC	3331

A.11.6.2.3.1	Test Purpose and Environment	3331

A.11.6.2.3.2	Test Parameters	3331

A.11.6.2.3.3	Test Requirements	3333

A.11.6.2.4	Inter-frequency measurement accuracy	3333

A.11.6.2.4.1	Test Purpose and Environment	3333

A.11.6.2.4.2	Test Parameters	3333

A.11.6.2.4.3	Test Requirements	3339

A.11.6.3	SS-SINR	3339

A.11.6.3.1	Intra-frequency measurement accuracy	3339

A.11.6.3.1.1	Test Purpose and Environment	3339

A.11.6.3.1.2	Test Parameters	3339

A.11.6.3.1.3	Test Requirements	3341

A.11.6.3.2	Inter-frequency measurement accuracy	3341

A.11.6.3.2.1	Test Purpose and Environment	3341

A.11.6.3.2.2	Test Parameters	3341

A.11.6.3.2.3	Test Requirements	3343

A.11.6.3.3	Intra-frequency measurement accuracy on SCC	3343

A.11.6.3.3.1	Test Purpose and Environment	3343

A.11.6.3.3.2	Test Parameters	3343

A.11.6.3.3.3	Test Requirements	3345

A.11.6.3.4	Inter-frequency measurement accuracy	3345

A.11.6.3.4.1	Test Purpose and Environment	3345

A.11.6.3.4.2	Test Parameters	3345

A.11.6.3.4.3	Test Requirements	3349

A.11.6.4	L1-RSRP measurement for beam reporting with CCA serving cell	3349

A.11.6.4.1	SSB based L1-RSRP measurement	3349

A.11.6.4.1.1	Test Purpose and Environment	3349

A.11.6.4.1.2	Test parameters	3349

A.11.6.4.1.3	Test Requirements	3351

A.11.6.5	RSSI	3351

A.11.6.5.1	Intra-frequency RSSI measurement accuracy on PCC with CCA	3351

A.11.6.5.1.1	Test Purpose and Environment	3351

A.11.6.5.1.2	Test parameters	3351

A.11.6.5.1.3	Test Requirements	3352

A.11.6.5.2	Intra-frequency RSSI measurement accuracy on SCC with CCA	3352

A.11.6.5.2.1	Test Purpose and Environment	3352

A.11.6.5.2.2	Test parameters	3352

A.11.6.5.2.3	Test Requirements	3354

A.11.6.5.3	Inter-frequency RSSI measurement accuracy on a carrier with CCA	3354

A.11.6.5.3.1	Test Purpose and Environment	3354

A.11.6.5.3.2	Test parameters	3354

A.11.6.5.3.3	Test Requirements	3356

A.11.6.6	Channel occupancy	3356

A.11.6.6.1	Intra-frequency channel occupancy measurement accuracy on PCC with CCA	3356

A.11.6.6.1.1	Test Purpose and Environment	3356

A.11.6.6.1.2	Test parameters	3356

A.11.6.6.1.3	Test Requirements	3357

A.11.6.6.2	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	3358

A.11.6.6.2.1	Test Purpose and Environment	3358

A.11.6.6.2.2	Test parameters	3358

A.11.6.6.2.3	Test Requirements	3359

A.11.6.6.3	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	3359

A.11.6.6.3.1	Test Purpose and Environment	3359

A.11.6.6.3.2	Test parameters	3360

A.11.6.6.3.3	Test Requirements	3361

A.11.6.7	E-UTRAN RSRP	3361

A.11.6.8	E-UTRAN RSRQ	3361

A.11.6.9	E-UTRAN SINR	3361

A.12	E-UTRA Standalone Tests with at Least One NR Cell under CCA	3361

A.12.1	RRC_IDLE state mobility	3362

A.12.1.1	Inter-RAT cell re-selection to NR on a carrier frequency with CCA	3362

A.12.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1 when target cell is subject to CCA	3362

A.12.1.1.1.1	Test Purpose and Environment	3362

A.12.1.1.1.2	Test Requirements	3365

A.12.2	RRC_CONNECTED state mobility	3365

A.12.2.1	Handover	3365

A.12.2.1.1	E-UTRAN - NR with CCA handover	3365

A.12.2.1.1.1	Test Purpose and Environment	3365

A.12.2.1.1.2	Test Requirements	3369

A.12.3	Void	3369

A.12.4	Measurement procedure	3369

A.12.4.1	E-UTRANNR inter-RAT SFTD measurements	3369

A.12.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay with NR under CCA in non-DRX	3369

A.12.4.1.1.1	Test Purpose and Environment	3369

A.12.4.1.1.2	Test Requirements	3371

A.12.4.2	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	3371

A.12.4.2.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	3371

A.12.4.2.1.1	Test Purpose and Environment	3371

A.12.4.2.1.2	Test Requirements	3375

A.12.4.2.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	3375

A.12.4.2.2.1	Test Purpose and Environment	3375

A.12.4.2.2.2	Test Requirements	3378

A.12.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	3378

A.12.4.2.3.1	Test Purpose and Environment	3378

A.12.4.2.3.2	Test Requirements	3382

A.12.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	3382

A.12.4.2.4.1	Test Purpose and Environment	3382

A.12.4.2.4.2	Test Requirements	3385

A.12.4.2.5	Void	3386

A.12.4.2.6	Void	3386

A.12.5	Measurement performance	3386

A.12.5.1	E-UTRANNR SFTD	3386

A.12.5.1.1	Inter-RAT SFTD accuracy with NR target cell under CCA	3386

A.12.5.1.1.1	Test Purpose	3386

A.12.5.1.1.2	Test Environment	3386

A.12.5.1.1.3	Test Requirements	3388

A.12.5.2	Void	3388

A.12.5.3	Void	3388

A.12.5.4	Void	3388

A.12.5.5	Void	3389

A.12.5.6	Void	3389

A.13	NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	3392

A.13.1	Void	3392

A.13.1.1	Void	3392

A.13.1.2	Void	3392

A.13.2	Signalling characteristics	3392

A.13.2.1	Void	3392

A.13.2.2	SCell activation and deactivation delay	3392

A.13.2.2.1	SCell Activation and Deactivation of known SCell under CCA, 160 ms SCell measurement cycle	3392

A.13.2.2.1.1	Test Purpose and Environment	3392

A.13.2.2.1.2	Test Requirements	3395

A.13.2.2.2 SCell Activation and Deactivation of known SCell under CCA, 640 ms SCell measurement cycle	3395

A.13.2.2.2.1	Test Purpose and Environment	3395

A.13.2.2.2.2	Test Requirements	3396

A.13.2.2.3	SCell Activation and Deactivation of unknown SCell under CCA	3396

A.13.2.2.3.1	Test Purpose and Environment	3396

A.13.2.2.3.2	Test Requirements	3396

A.13.2.3	Void	3397

A.13.3	Measurement procedure	3397

A.13.3.1	Intra-frequency measurements	3397

A.13.3.1.1	Event-triggered reporting tests on SCC without gaps under non-DRX	3397

A.13.3.1.1.1	Test purpose and environment	3397

A.13.3.1.1.2	Test parameters	3397

A.13.3.1.1.3	Test Requirements	3400

A.13.3.1.2	Event-triggered reporting tests on SCC without gaps under DRX	3400

A.13.3.1.2.1	Test purpose and environment	3400

A.13.3.1.2.2	Test parameters	3400

A.13.3.1.2.3	Test Requirements	3403

A.13.3.1.3	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	3403

A.13.3.1.3.1	Test purpose and environment	3403

A.13.3.1.3.2	Test parameters	3403

A.13.3.1.3.3	Test Requirements	3406

A.13.3.1.4	Event-triggered reporting tests on SCC with per-UE gaps under DRX	3407

A.13.3.1.4.1	Test purpose and environment	3407

A.13.3.1.4.2	Test parameters	3407

A.13.3.1.4.3	Test Requirements	3410

A.13.3.1.5	Void	3410

A.13.3.1.6	Void	3410

A.13.3.2	Inter-frequency measurements	3410

A.13.3.2.1	Void	3410

A.13.3.2.2	Void	3410

A.13.3.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	3410

A.13.3.2.3.1	Test Purpose and Environment	3410

A.13.3.2.3.2	Test Requirements	3413

A.13.3.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	3414

A.13.3.2.4.1	Test Purpose and Environment	3414

A.13.3.2.4.2	Test Requirements	3417

A.13.3.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	3417

A.13.3.2.5.1	Test Purpose and Environment	3417

A.13.3.2.5.2	Test Requirements	3420

A.13.3.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	3420

A.13.3.2.6.1	Test Purpose and Environment	3420

A.13.3.2.6.2	Test Requirements	3424

A.13.3.3	L1-RSRP measurements for beam reporting	3424

A.13.3.3.1	SSB based L1-RSRP measurement when DRX is not used	3424

A.13.3.3.1.1	Test Purpose and Environment	3424

A.13.3.3.1.2	Test parameters	3424

A.13.3.3.1.3	Test Requirements	3426

A.13.3.3.2	SSB based L1-RSRP measurement when DRX is used	3427

A.13.3.3.2.1	Test Purpose and Environment	3427

A.13.3.3.2.2	Test parameters	3427

A.13.3.3.2.3	Test Requirements	3429

A.13.4	Measurement performance	3429

A.13.4.1	SS-RSRP	3429

A.13.4.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	3429

A.13.4.1.1.1	Test Purpose and Environment	3429

A.13.4.1.1.2	Test parameters	3429

A.13.4.1.1.3	Test Requirements	3431

A.13.4.2	SS-RSRQ	3431

A.13.4.2.1	Intra-frequency measurement accuracy on SCC	3431

A.13.4.2.1.1	Test Purpose and Environment	3431

A.13.4.2.1.2	Test Parameters	3431

A.13.4.2.1.3	Test Requirements	3435

A.13.4.3	SS-SINR	3435

A.13.4.3.1	Intra-frequency measurement accuracy on SCC	3435

A.13.4.3.1.1	Test Purpose and Environment	3435

A.13.4.3.1.2	Test Parameters	3435

A.13.4.3.1.3	Test Requirements	3439

A.13.4.4	L1-RSRP measurement for beam reporting with CCA serving cell	3439

A.13.4.4.1	SSB based L1-RSRP measurement	3439

A.13.4.4.1.1	Test Purpose and Environment	3439

A.13.4.4.1.2	Test parameters	3440

A.13.4.4.1.3	Test Requirements	3442

A.13.4.5	RSSI	3442

A.13.4.5.1 	Intra-frequency RSSI measurement accuracy on a carrier with CCA	3442

A.13.4.5.1.1	Test Purpose and Environment	3442

A.13.4.5.1.2	Test parameters	3442

A.13.4.5.1.3	Test Requirements	3444

A.13.4.5.2	Inter-frequency RSSI measurement accuracy on a carrier with CCA	3444

A.13.4.5.2.1	Test Purpose and Environment	3444

A.13.4.5.2.2	Test parameters	3444

A.13.4.5.2.3	Test Requirements	3446

A.13.4.6	Channel occupancy	3446

A.13.4.6.1	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	3446

A.13.4.6.1.1	Test Purpose and Environment	3446

A.13.4.6.1.2	Test parameters	3446

A.13.4.6.1.3	Test Requirements	3448

A.13.4.6.2	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	3448

A.13.4.6.2.1	Test Purpose and Environment	3448

A.13.4.6.2.2	Test parameters	3448

A.13.4.6.2.3	Test Requirements	3449

A.14	NR standalone tests for Satellite access	3450

A.14.1	RRC_IDLE state mobility	3450

A.14.1.1	Cell reselection to FR1 intra-frequency NR case	3450

A.14.1.1.1	Test Purpose and Environment	3450

A.14.1.1.2	Test Parameters	3450

A.14.1.1.3	Test Requirements	3451

A.14.1.2	Cell reselection to FR1 intra-frequency NR cell for UE configured with the feature for enhanced requirements	3452

A.14.1.2.1	Test Purpose and Environment	3452

A.14.1.2.2	Test Parameters	3452

A.14.1.2.3	Test Requirements	3453

A.14.1.3	Time-based measurement initiation to FR1 intra-frequency NR cell reselection	3454

A.14.1.3.1	Test Purpose and Environment	3454

A.14.1.3.2	Test Parameters	3454

A.14.1.3.3	Test Requirements	3455

A.14.1.4	Location-based measurement initiation to FR1 intra-frequency NR cell reselection	3456

A.14.1.4.1	Test Purpose and Environment	3456

A.14.1.4.2	Test Parameters	3456

A.14.1.4.3	Test Requirements	3457

A.14.1.5	Cell reselection to FR1 inter-frequency NR case	3458

A.14.1.5.1	Test Purpose and Environment	3458

A.14.1.5.2	Test Parameters	3458

A.14.1.5.3	Test Requirements	3460

A.14.1.6	Cell re-selection to FR1 inter-frequency NR cell for UE configured with feature for enhanced requirements	3460

A.14.1.6.1	Test Purpose and Environment	3460

A.14.1.6.2	Test Parameters	3460

A.14.1.6.3	Test Requirements	3462

A.14.1.7	Time-based measurement initiation to FR1 inter-frequency cell reselection	3462

A.14.1.7.1	Test Purpose and Environment	3462

A.14.1.7.2	Test Parameters	3463

A.14.1.7.3	Test Requirements	3464

A.14.1.8	Location-based measurement initiation to FR1 inter-frequency NR cell reselection	3464

A.14.1.8.1	Test Purpose and Environment	3464

A.14.1.8.2	Test Parameters	3464

A.14.1.8.3	Test Requirements	3466

A.14.1.9	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	3466

A.14.1.9.1	Test Purpose and Environment	3466

A.14.1.9.2	Test Parameters	3466

A.14.1.9.3	Test Requirements	3468

A.14.1.10	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	3469

A.14.1.10.1	Test Purpose and Environment	3469

A.14.1.10.2	Test Parameters	3469

A.14.1.10.3	Test Requirements	3470

A.14.1.11	Cell reselection to FR1 inter-RAT for NR NTN carrier	3471

A.14.1.11.1	Test purpose and Environment	3471

A.14.1.11.2	Test parameters	3471

A.14.1.11.3	Test requirements	3473

A.14.1.12	Cell re-selection to FR1 inter-frequency NR case with TN carrier	3473

A.14.1.12.1	Test purpose and Environment	3474

A.14.1.12.2	Test parameters	3474

A.14.1.12.3	Test requirements	3476

A.14.2	RRC_CONNECTED state mobility	3476

A.14.2.1	Handover	3476

A.14.2.1.1	Intra-frequency SAN Handover from FR1 to FR1	3476

A.14.2.1.1.1	Test Purpose and Environment	3476

A.14.2.1.1.2	Test Parameters	3476

A.14.2.1.1.3	Test Requirements	3478

A.14.2.1.2	Inter-frequency SAN Handover from FR1 to FR1	3478

A.14.2.1.2.1	Test Purpose and Environment	3478

A.14.2.1.2.2	Test Parameters	3478

A.14.2.1.2.3	Test Requirements	3480

A.14.2.1.3	Intra-frequency SAN time-based conditional Handover from FR1 to FR1	3480

A.14.2.1.3.1	Test Purpose and Environment	3480

A.14.2.1.3.2	Test Parameters	3480

A.14.2.1.3.3	Test Requirements	3482

A.14.2.1.4	Inter-frequency SAN time-based conditional Handover from FR1 to FR1	3482

A.14.2.1.4.1	Test Purpose and Environment	3482

A.14.2.1.4.2	Test Parameters	3482

A.14.2.1.4.3	Test Requirements	3484

A.14.2.1.5	Intra-frequency SAN distance-based conditional Handover from FR1 to FR1	3485

A.14.2.1.5.1	Test Purpose and Environment	3485

A.14.2.1.5.2	Test Parameters	3485

A.14.2.1.5.3	Test Requirements	3486

A.14.2.1.6	Inter-frequency SAN distance-based conditional Handover from FR1 to FR1	3487

A.14.2.1.6.1	Test Purpose and Environment	3487

A.14.2.1.6.2	Test Parameters	3487

A.14.2.1.6.3	Test Requirements	3489

A.14.2.1.7	Intra-frequency intra-satellite Handover from FR2-NTN to FR2-NTN	3489

A.14.2.1.7.1	Test Purpose and Environment	3489

A.14.2.1.7.2	Test Parameters	3490

A.14.2.1.7.3	Test Requirements	3491

A.14.2.1.8	Intra-frequency SAN Handover from FR1 to FR1	3492

A.14.2.1.8.1	Test Purpose and Environment	3492

A.14.2.1.8.2	Test Parameters	3492

A.14.2.1.8.3	Test Requirements	3493

A.14.2.1.9	Intra-frequency inter-satellite handover from FR2-NTN to FR2-NTN	3494

A.14.2.1.9.1	Test Purpose and Environment	3494

A.14.2.1.9.2	Test Parameters	3494

A.14.2.1.9.3	Test Requirements	3496

A.14.2.1.10	Intra-frequency SAN Handover from FR1 to FR1 for UE operating on a cell with less than 5 MHz BWA.14.2.1.10.1 Test Purpose and Environment	3496

A.14.2.1.10.2	Test Parameters	3496

A.14.2.1.10.3	Test Requirements	3498

A.14.2.1.11	Intra-frequency SAN time-based conditional Handover from FR1 to FR1 for UE operating on a cell with less than 5 MHz BW	3498

A.14.2. 11.1	Test Purpose and Environment	3498

A.14.2. 11.2	Test Parameters	3498

A.14.2. 11.3	Test Requirements	3500

A.14.2.2	RRC Connection Mobility Control	3500

A.14.2.2.1	SA: RRC Re-establishment for SAN	3500

A.14.2.2.1.1	Intra-frequency RRC Re-establishment in FR1	3500

A.14.2.2.1.2	Inter-frequency RRC Re-establishment in FR1	3502

A.14.2.2.2	Random Access	3505

A.14.2.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	3505

A.14.2.2.2.1.1	Test Purpose and Environment	3505

A.14.2.2.2.1.2	Test Requirements	3506

A.14.2.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone	3507

A.14.2.2.2.2.1	Test Purpose and Environment	3507

A.14.2.2.2.2.2	Test Requirements	3509

A.14.2.2.3	RRC Connection Release with Redirection	3510

A.14.2.2.3.1	Redirection from NR in FR1 to NR in FR1	3510

A.14.2.2.3.1.1	Test Purpose and Environment	3510

A.14.2.2.3.1.2	Test Parameters	3510

A.14.2.2.3.1.3	Test Requirements	3512

A.14.2.2.4	RACH-based Hard Satellite switching with re-synchronization from FR1 to FR1	3513

A.14.2.2.4.1	Test Purpose and Environment	3513

A.14.2.2.4.2	Test Parameters	3513

A.14.2.2.4.3	Test Requirements	3515

A.14.2.2.5	RACH-less Soft Satellite switching with re-synchronization from FR1 to FR1	3515

A.14.2.2.5.1	Test Purpose and Environment	3515

A.14.2.2.5.2	Test Parameters	3515

A.14.2.2.5.3	Test Requirements	3517

A.14.2.2.6	RACH-based hard Satellite switching with re-synchronization from FR1 to FR1 for less than 5MHz with NTN	3517

A.14.2.2.6.1	Test Purpose and Environment	3517

A.14.2.2.6.2	Test Parameters	3517

A.14.2.2.6.3	Test Requirements	3518

A.14.2.3	Intra-frequency SAN time-based conditional Handover without L3 measurement criteria from FR1 to FR1	3518

A.14.2.3.1	Test Purpose and Environment	3519

A.14.2.3.2	Test Parameters	3519

A.14.2.3.3	Test Requirements	3520

A.14.2.4	Inter-frequency SAN time-based conditional Handover without L3 measurement criteria from FR1 to FR1	3520

A.14.2.4.1	Test Purpose and Environment	3521

A.14.2.4.2	Test Parameters	3521

A.14.2.4.3	Test Requirements	3522

A.14.3	Timing for Satellite Access	3523

A.14.3.1	UE transmit timing for Satellite Access	3523

A.14.3.1.1	NR UE Transmit Timing Test for FR1	3523

A.14.3.1.1.1	Test Purpose and environment	3523

A.14.3.1.1.2	Test requirements	3524

A.14.3.1.2	NR UE Transmit Timing Test for FR2-NTN	3525

A.14.3.1.2.1	Test Purpose and environment	3525

A.14.3.1.2.2	Test requirements	3528

A.14.3.2	Timing advance for satellite access	3528

A.14.3.2.1	SA FR1 timing advance adjustment accuracy	3528

A.14.3.2.1.1	Test Purpose and Environment	3528

A.14.3.2.1.2	Test Parameters	3528

A.14.3.2.1.3	Test Requirements	3531

A.14.3.2.3	SA FR2-NTN timing advance adjustment accuracy	3531

A.14.3.2.3.1	Test Purpose and Environment	3531

A.14.3.2.3.2	Test Parameters	3531

A.14.3.2.1.3	Test Requirements	3534

A.14.4	Signalling characteristics	3534

A.14.4.1	Radio link Monitoring	3534

A.14.4.1.1	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode	3534

A.14.4.1.1.1	Test Purpose and Environment	3534

A.14.4.1.1.2	Test Requirements	3536

A.14.4.1.2	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode	3537

A.14.4.1.2.1	Test Purpose and Environment	3537

A.14.4.1.2.2	Test Requirements	3539

A.14.4.1.3	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode	3539

A.14.4.1.3.1	Test Purpose and Environment	3539

A.14.4.1.3.2	Test Requirements	3541

A.14.4.1.4	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode	3542

A.14.4.1.4.1	Test Purpose and Environment	3542

A.14.4.1.4.2	Test Requirements	3544

A.14.4.1.5	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in non-DRX mode	3544

A.14.4.1.5.1	Test Purpose and Environment	3544

A.14.4.1.5.2	Test Requirements	3547

A.14.4.1.6	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in non-DRX mode	3547

A.14.4.1.6.1	Test Purpose and Environment	3547

A.14.4.1.6.2	Test Requirements	3550

A.14.4.1.7	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in DRX mode	3550

A.14.4.1.7.1	Test Purpose and Environment	3550

A.14.4.1.7.2	Test Requirements	3553

A.14.4.1.8	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in DRX mode	3553

A.14.4.1.8.1	Test Purpose and Environment	3553

A.14.4.1.8.2	Test Requirements	3556

A.14.4.1.9	Radio Link Monitoring Out-of-sync Test for FR2 SAN PCell configured with SSB-based RLM RS in non-DRX mode	3556

A.14.4.1.9.1	Test Purpose and Environment	3556

A.14.4.1.9.2	Test Requirements	3558

A.14.4.1.10	Radio Link Monitoring In-sync Test for FR2 SAN PCell configured with SSB-based RLM RS in non-DRX mode	3559

A.14.4.1.10.1	Test Purpose and Environment	3559

A.14.4.1.10.2	Test Requirements	3561

A.14.4.1.11	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode	3561

A.14.4.1.11.1	Test Purpose and Environment	3561

A.14.4.1.11.2	Test Requirements	3562

A.14.4.1.12	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode for less than 5 MHz BW	3562

A.14.4.1.12.1	Test Purpose and Environment	3562

A.14.4.1.12.2	Test Requirements	3563

A.14.4.2	Beam Failure Detection and Link recovery procedures for satellite access	3564

A.14.4.2.1	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in non-DRX mode	3564

A.14.4.2.1.1	Test Purpose and Environment	3564

A.14.4.2.1.2	Test Requirements	3566

A.14.4.2.2	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in DRX mode	3567

A.14.4.2.2.1	Test Purpose and Environment	3567

A.14.4.2.2.2	Test Requirements	3569

A.14.4.2.3	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS-based BFD and LR in non-DRX mode	3570

A.14.4.2.3.1	Test Purpose and Environment	3570

A.14.4.2.3.2	Test Requirements	3572

A.14.4.2.4	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS-based BFD and LR in DRX mode	3573

A.14.4.2.4.1	Test Purpose and Environment	3573

A.14.4.2.4.2	Test Requirements	3575

A.14.4.2.5	Void	3576

A.14.4.2.6	Void	3576

A.14.4.2.7	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in non-DRX mode for a UE operating on a cell with less than 5 MHz BW	3576

A.14.4.2.7.1	Test Purpose and Environment	3576

A.14.4.2.7.2	Test Requirements	3577

A.14.4.3	Active BWP switch for satellite access	3577

A.14.4.3.1	DCI-based and Timer-based Active BWP Switch	3577

A.14.4.3.1.1	NR FR1 DL active BWP switch with non-DRX in SA	3577

A.14.4.3.1.1.1	Test Purpose and Environment	3577

A.14.4.3.1.1.2	Test Requirements	3579

A.14.4.3.2	RRC-based Active BWP Switch	3580

A.14.4.3.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	3580

A.14.4.3.2.1.1	Test Purpose and Environment	3580

A.14.4.3.2.1.2	Test Requirements	3581

A.14.4.4	UE specific CBW change for satellite access	3582

A.14.4.4.1	UE specific CBW change on PCell in FR1 in non-DRX	3582

A.14.4.4.1.1	Test Purpose and Environment	3582

A.14.4.4.1.2	Test Requirements	3584

A.14.4.5	Pathloss reference signal switching delay	3584

A.14.4.5.1	MAC-CE based pathloss reference signal switch delay	3584

A.14.4.5.1.1	Test Purpose and Environment	3584

A.14.4.5.1.2	Test Requirements	3586

A.14.5	Measurement procedure	3586

A.14.5.1	Intra-frequency Measurements	3586

A.14.5.1.1	SA event triggered reporting tests without gap under non-DRX	3587

A.14.5.1.1.1	Test purpose and Environment	3587

A.14.5.1.1.2	Test parameters	3587

A.14.5.1.1.3	Test Requirements	3588

A.14.5.1.2	SA event triggered reporting tests without gap under DRX	3588

A.14.5.1.2.1	Test purpose and Environment	3588

A.14.5.1.2.2	Test parameters	3588

A.14.5.1.2.3	Test Requirements	3590

A.14.5.1.3	SA event triggered reporting tests without gap under non-DRX with SSB index reading	3590

A.14.5.1.3.1	Test purpose and Environment	3590

A.14.5.1.3.2	Test parameters	3590

A.14.5.1.3.3	Test Requirements	3592

A.14.5.1.4	SA event triggered reporting tests with single measurement gap under non-DRX for satellite access	3592

A.14.5.1.4.1	Test purpose and Environment	3592

A.14.5.1.4.2	Test parameters	3592

A.14.5.1.4.3	Test Requirements	3594

A.14.5.1.5	SA event triggered reporting tests with FNO concurrent gaps under DRX for satellite access	3594

A.14.5.1.5.1	Test purpose and Environment	3594

A.14.5.1.5.2	Test parameters	3594

A.15.5.1.5.3	Test Requirements	3596

A.14.5.1.6	SA event triggered reporting tests with PPO concurrent gaps under non-DRX with SSB index reading for satellite access	3596

A.14.5.1.6.1	Test purpose and Environment	3596

A.14.5.1.6.2	Test parameters	3596

A.14.5.1.6.3	Test Requirements	3598

A.14.5.1.7	SA event triggered reporting test with SSB time index reading without gap under non-DRX for FR2-NTN	3598

A.14.5.1.7.1	Test purpose and Environment	3598

A.14.5.1.7.2	Test parameters	3598

A.14.5.1.7.3	Test Requirements	3600

A.14.5.1.8	SA event triggered reporting tests without gap under non-DRX with SSB index reading under less 5MHz BW	3600

A.14.5.1.8.1	Test purpose and Environment	3600

A.14.5.1.8.2	Test parameters	3600

A.14.5.1.8.3	Test Requirements	3602

A.14.5.2	Inter-frequency Measurements	3602

A.14.5.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with single gap for satellite access	3602

A.14.5.2.1.1	Test Purpose and Environment	3602

A.14.5.2.1.2	Test Requirements	3604

A.14.5.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used with single gap for satellite access	3604

A.14.5.2.2.1	Test Purpose and Environment	3604

A.14.5.2.2.2	Test Requirements	3607

A.14.5.2.3	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used with single gap for satellite access	3607

A.14.5.2.3.1	Test Purpose and Environment	3607

A.14.5.2.3.2	Test Requirements	3609

A.14.5.2.4	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in fully non-overlapped for satellite access	3609

A.14.5.2.4.1	Test Purpose and Environment	3609

A.14.5.2.4.2	Test Requirements	3611

A.14.5.2.5	void	3611

A.14.5.2.5.1	void	3611

A.14.5.2.5.2	void	3611

A.14.5.2.6	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in partially partial overalpping for satellite access	3612

A.14.5.2.6.1	Test Purpose and Environment	3612

A.14.5.2.6.2	Test Requirements	3614

A.14.5.2.7	Event triggered reporting test without gap under non-DRX	3614

A.14.5.2.7.1	Test purpose and Environment	3614

A.14.5.2.7.2	Test parameters	3614

A.14.5.2.7.3	Test Requirements	3615

A.14.5.2.8	Event triggered reporting tests without gap under DRX	3615

A.14.5.2.8.1	Test purpose and Environment	3615

A.14.5.2.8.2	Test parameters	3616

A.14.5.2.8.3	Test Requirements	3617

A.14.5.2.9	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used with single gap for 3 MHz channel bandwidth in satellite access	3617

A.14.5.2.9.1	Test Purpose and Environment	3617

A.14.5.2.9.2	Test Requirements	3619

A.14.5.3	L1-RSRP measurement for beam reporting for satellite access	3619

A.14.5.3.1	SSB based L1-RSRP measurement for satellite access when DRX is not used	3619

A.14.5.3.1.1	Test Purpose and Environment	3619

A.14.5.3.1.2	Test parameters	3619

A.14.5.3.1.3	Test Requirements	3621

A.14.5.3.2	SSB based L1-RSRP measurement for satellite access when DRX is used	3621

A.14.5.3.2.1	Test Purpose and Environment	3621

A.14.5.3.2.2	Test parameters	3621

A.14.5.3.2.3	Test Requirements	3623

A.14.5.3.3	CSI-RS based L1-RSRP measurement for satellite access when DRX is not used	3623

A.14.5.3.3.1	Test Purpose and Environment	3623

A.14.5.3.3.2	Test parameters	3623

A.14.5.3.3.3	Test Requirements	3625

A.14.5.3.4	CSI-RS based L1-RSRP measurement for satellite access when DRX is used	3625

A.14.5.3.4.1	Test Purpose and Environment	3625

A.14.5.3.4.2	Test parameters	3625

A.14.5.3.4.3	Test Requirements	3627

A.14.5.3.5	SSB based L1-RSRP measurement when DRX is not used in FR2-NTN	3627

A.14.5.3.5.1	Test Purpose and Environment	3627

A.14.5.3.5.2	Test parameters	3627

A.14.5.3.5.3	Test Requirements	3629

A.14.6	Measurement Performance requirements	3629

A.14.6.1	SS-RSRP for SAN	3629

A.14.6.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	3629

A.14.6.1.1.1	Test Purpose and Environment	3629

A.14.6.1.1.2	Test parameters	3629

A.14.6.1.1.3	Test Requirements	3630

A.14.6.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	3630

A.14.6.1.2.1	Test Purpose and Environment	3630

A.14.6.1.2.2	Test parameters	3631

A.14.6.1.2.3	Test Requirements	3632

A.14.6.1.3	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3632

A.14.6.1.3.1	Test Purpose and Environment	3632

A.14.6.1.3.2	Test parameters	3632

A.14.6.1.3.3	Test Requirements	3634

A.14.6.2	SS-RSRQ	3634

A.14.6.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for satellite access	3634

A.14.6.2.1.1	Test Purpose and Environment	3634

A.14.6.2.1.2	Test Parameters	3634

A.14.6.2.1.3	Test Requirements	3636

A.14.6.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for satellite access	3636

A.14.6.2.2.1	Test Purpose and Environment	3636

A.14.6.2.2.2	Test Parameters	3636

A.14.6.2.2.3	Test Requirements	3637

A.14.6.3	SS-SINR	3637

A.14.6.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	3637

A.14.6.3.1.1	Test Purpose and Environment	3637

A.14.6.3.1.2	Test Parameters	3637

A.14.6.3.1.3	Test Requirements	3639

A.14.6.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	3640

A.14.6.3.2.1	Test Purpose and Environment	3640

A.14.6.3.2.2	Test Parameters	3640

A.14.6.3.2.3	Test Requirements	3641

A.14.6.4	L1-RSRP measurement for beam reporting	3641

A.14.6.4.1	SSB based L1-RSRP measurement	3641

A.14.6.4.1.1	Test Purpose and Environment	3641

A.14.6.4.1.2	Test parameters	3641

A.14.6.4.1.3	Test Requirements	3643

A.14.6.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	3643

A.14.6.4.2.1	Test Purpose and Environment	3643

A.14.6.4.2.2	Test parameters	3643

A.14.6.4.2.3	Test Requirements	3644

A.14.6.4.3	SSB based L1-RSRP measurement for VSAT UE in FR2-NTN when DRX is not used	3644

A.14.6.4.3.1	Test Purpose and Environment	3644

A.14.6.4.3.2	Test parameters	3644

A.14.6.4.3.3	Test Requirements	3646

A.15	NR standalone tests with one or more NR cells in FR2-2	3648

A.15.1	SA: RRC_IDLE state mobility	3648

A.15.1.1	Cell re-selection to NR	3648

A.15.1.1.1	Cell re-selection to FR2-2 intra-frequency NR case	3648

A.15.1.1.1.1	Test Purpose and Environment	3648

A.15.1.1.1.2	Test Parameters	3648

A.15.1.1.1.3	Test Requirements	3650

A.15.1.2	Cell re-selection to FR2-2 inter-frequency NR case	3650

A.15.1.2.1	Test Purpose and Environment	3650

A.15.1.2.2	Test Parameters	3651

A.15.1.2.3	Test Requirements	3653

A.15.1.3	Cell re-selection to FR2-2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	3653

A.15.1.3.1	Test Purpose and Environment	3653

A.15.1.3.2	Test Parameters	3653

A.15.1.3.3	Test Requirements	3655

A.15.1.4	Cell re-selection to FR2-2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	3656

A.15.1.4.1	Test Purpose and Environment	3656

A.15.1.4.2	Test Parameters	3656

A.15.1.4.3	Test Requirements	3658

A.15.1.5	Cell re-selection to FR2-2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	3658

A.15.1.5.1	Test Purpose and Environment	3658

A.15.1.5.2	Test Parameters	3658

A.15.1.5.3	Test Requirements	3660

A.15.1.6	Cell re-selection to FR2-2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	3661

A.15.1.6.1	Test Purpose and Environment	3661

A.15.1.6.2	Test Parameters	3661

A.15.1.6.3	Test Requirements	3663

A.15.2	Signaling characteristics	3663

A.15.2.1	SCell Activation and Deactivation Delay	3663

A.15.2.1.1	SCell Activation and deactivation for SCell in FR2-2 intra-band in non-DRX	3663

A.15.2.1.1.1	Test Purpose and Environment	3663

A.15.2.1.1.2	Test Requirements	3665

A.15.2.1.2	SCell Activation and deactivation for FR1+FR2-2 inter-band with target SCell in FR2-2	3665

A.15.2.1.2.1	Test Purpose and Environment	3665

A.15.2.1.2.2	Test Requirements	3668

A.15.2.1.3	SCell Activation and deactivation for SCell in FR2-2 inter-band in non-DRX	3668

A.15.2.1.3.1	Test Purpose and Environment	3669

A.15.2.1.3.2	Test Requirements	3671

A.15.2.1.4	Direct SCell activation at SCell addition of known SCell in FR2-2	3672

A.15.2.1.4.1	Test Purpose and Environment	3672

A.15.2.1.4.2	Test Requirements	3674

A.15.2.1.5	Direct SCell activation at handover with known SCell in FR2-2	3675

A.15.2.1.5.1	Test Purpose and Environment	3675

A.15.2.1.5.2	Test Requirements	3677

A.15.3	RRC_CONNECTED state mobility	3678

A.15.3.1	Handover	3678

A.15.3.1.1	Intra-frequency handover from FR2-2 carrier with CCA to FR2-2 carrier with CCA; unknown target cell	3678

A.15.3.1.1.1	Test Purpose and Environment	3678

A.15.3.1.1.2	Test Parameters	3678

A.15.3.1.1.3	Test Requirements	3680

A.15.3.1.2	Inter-frequency handover from FR1 to FR2-2 carrier with CCA; unknown target cell	3681

A.15.3.1.2.1	Test Purpose and Environment	3681

A.15.3.1.2.2	Test Parameters	3681

A.15.3.1.2.3	Test Requirements	3683

A.15.4	Measurement procedure	3683

A.15.4.1	Intra-frequency Measurements	3683

A.15.4.1.1	SA event triggered reporting test without gap under non-DRX for FR2-2 with CCA	3683

A.15.4.1.1.1	Test purpose and Environment	3683

A.15.4.1.1.2	Test Requirements	3686

A.15.4.2	Inter-frequency Measurements	3687

A.15.4.2.1	SA event triggered reporting tests for FR2-2 with CCA without SSB time index detection when DRX is not used (PCell in FR2-2)	3687

A.15.4.2.1.1	Test Purpose and Environment	3687

A.15.4.2.1.2	Test Requirements	3689

A.16	NR standalone tests with all NR cells in FR1 for RedCap	3690

A.16.1	SA: RRC_IDLE state mobility for RedCap	3690

A.16.1.1	Cell re-selection to NR	3690

A.16.1.1.1	Cell re-selection to FR1 intra-frequency NR case for 1 Rx UE	3690

A.16.1.1.1.1	Test Purpose and Environment	3690

A.16.1.1.1.2	Test Parameters	3690

A.16.1.1.1.3	Test Requirements	3692

A.16.1.1.2	Cell re-selection to FR1 intra-frequency NR case for 2 Rx UE	3693

A.16.1.1.2.1	Test Purpose and Environment	3693

A.16.1.1.2.2	Test Parameters	3693

A.16.1.1.2.3	Test Requirements	3695

A.16.1.1.3	Cell re-selection to FR1 inter-frequency NR case for 1 Rx UE	3695

A.16.1.1.3.1	Test Purpose and Environment	3695

A.16.1.1.3.2	Test Parameters	3695

A.16.1.1.3.3	Test Requirements	3698

A.16.1.1.4	Cell re-selection to FR1 inter-frequency NR case for 2 Rx UE	3698

A.16.1.1.4.1	Test Purpose and Environment	3698

A.16.1.1.4.2	Test Parameters	3698

A.16.1.1.4.3	Test Requirements	3701

A.16.1.1.5	Cell re-selection to FR1 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	3701

A.16.1.1.5.1	Test Purpose and Environment	3701

A.16.1.1.5.2	Test Parameters	3701

A.16.1.1.5.3	Test Requirements	3704

A.16.1.1.6	Cell re-selection to FR1 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	3704

A.16.1.1.6.1	Test Purpose and Environment	3704

A.16.1.1.6.2	Test Parameters	3704

A.16.1.1.6.3	Test Requirements	3706

A.16.1.1.7	Cell re-selection to FR1 inter-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	3707

A.16.1.1.7.1	Test Purpose and Environment	3707

A.16.1.1.7.2	Test Parameters	3707

A.16.1.1.7.3	Test Requirements	3709

A.16.1.1.8	Cell re-selection to FR1 inter-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	3710

A.16.1.1.8.1	Test Purpose and Environment	3710

A.16.1.1.8.2	Test Parameters	3710

A.16.1.1.8.3	Test Requirements	3712

A.16.1.2	Inter-RAT E-UTRAN cell re-selection for RedCap	3713

A.16.1.2.1	Cell re-selection to higher priority E-UTRAN for 1 RX	3713

A.16.1.2.1.1	Test Purpose and Environment	3713

A.16.1.2.1.2	Test Parameters	3713

A.16.1.2.1.3	Test Requirements	3716

A.16.1.2.2	Cell re-selection to higher priority E-UTRAN for 2 RX	3716

A.16.1.2.2.1	Test Purpose and Environment	3716

A.16.1.2.2.2	Test Parameters	3716

A.16.1.2.2.3	Test Requirements	3719

A.16.1.2.3.1	Test Purpose and Environment	3719

A. 16.1.2.3.2	Test Parameters	3719

A.16.1.2.3.3	Test Requirements	3722

A.16.1.2.4.1	Test Purpose and Environment	3722

A.16.1.2.4.2	Test Parameters	3722

A.16.1.3.1.3	Test Requirements	3725

A.16.1.2.5	Cell re-selection to lower priority E-UTRAN for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	3725

A.16.1.2.5.1	Test Purpose and Environment	3725

A.16.1.2.5.2	Test Parameters	3725

A.16.1.2.5.3	Test Requirements	3728

A.16.1.2.6	Cell re-selection to lower priority E-UTRAN for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	3728

A.16.1.2.6.1	Test Purpose and Environment	3728

A.16.1.2.6.2	Test Parameters	3729

A.16.1.2.6.3	Test Requirements	3731

A.16.2	SA: RRC_INACTIVE state mobility for RedCap	3732

A.16.2.1	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap	3732

A.16.2.1.1	NR UE CG-SDT Test in FR1 for 1 Rx RedCap UE	3732

A.16.2.1.1.1	Test purpose and Environment	3732

A.16.2.1.1.2	Test Parameters	3733

A.16.2.1.1.3	Test requirements	3735

A.16.2.1.2	NR UE CG-SDT Test in FR1 for 2 Rx RedCap UE	3735

A.16.2.1.2.1	Test purpose and Environment	3735

A.16.2.1.2.2	Test Parameters	3737

A.16.2.1.2.3	Test requirements	3738

A.16.2.2	Cell Reselection for Positioning	3739

A.16.2.2.1	Cell re-selection to FR1 intra-frequency NR case with RRC_INACTIVE eDRX and positioning SRS	3739

A.16.2.2.1.1	Test Purpose and Environment	3739

A.16.2.2.1.2	Test Parameters	3739

A.16.2.2.1.3	Test Requirements	3742

A.16.3	RRC_CONNECTED state mobility for RedCap	3742

A.16.3.1	Handover	3742

A.16.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell for 1 Rx UE	3742

A.16.3.1.1.1	Test Purpose and Environment	3742

A.16.3.1.1.2	Test Parameters	3742

A.16.3.1.1.3	Test Requirements	3744

A.16.3.1.2	Intra-frequency handover from FR1 to FR1; known target cell for 2 Rx UE	3744

A.16.3.1.2.1	Test Purpose and Environment	3744

A.16.3.1.2.2	Test Parameters	3744

A.16.3.1.2.3	Test Requirements	3746

A.16.3.1.3	Intra-frequency handover from FR1 to FR1; unknown target cell for 1 Rx UE	3746

A.16.3.1.3.1	Test Purpose and Environment	3746

A.16.3.1.3.2	Test Parameters	3747

A.16.3.1.3.3	Test Requirements	3749

A.16.3.1.4	Intra-frequency handover from FR1 to FR1; unknown target cell for 2 Rx UE	3749

A.16.3.1.4.1	Test Purpose and Environment	3749

A.16.3.1.4.2	Test Parameters	3749

A.16.3.1.5	Inter-frequency handover from FR1 to FR1; unknown target cell for 1 Rx UE	3751

A.16.3.1.5.1	Test Purpose and Environment	3751

A.16.3.1.5.2	Test Parameters	3751

A.16.3.1.5.3	Test Requirements	3753

A.16.3.1.6	Inter-frequency handover from FR1 to FR1; unknown target cell for 2 Rx UE	3754

A.16.3.1.6.1	Test Purpose and Environment	3754

A.16.3.1.6.2	Test Parameters	3754

A.16.3.1.6.3	Test Requirements	3756

A.16.3.1.7	SA NR - E-UTRAN handover for 1 Rx UE	3756

A.16.3.1.7.1	Test Purpose and Environment	3756

A.16.3.1.7.2	Test Requirements	3760

A.16.3.1.8	SA NR - E-UTRAN handover for 2 Rx UE	3760

A.16.3.1.8.1	Test Purpose and Environment	3760

A.16.3.1.8.2	Test Requirements	3763

A.16.3.1.9	SA NR - E-UTRAN handover with unknown target cell for 1 Rx UE	3763

A.16.3.1.9.1	Test Purpose and Environment	3763

A.16.3.1.9.2	Test Requirements	3766

A.16.3.1.10	SA NR - E-UTRAN handover with unknown target cell for 2 Rx UE	3766

A.16.3.1.10.1	Test Purpose and Environment	3767

A.16.3.1.10.2	Test Requirements	3770

A.16.3.2	RRC Connection Mobility Control	3770

A.16.3.2.1	SA: RRC Re-establishment	3770

A.16.3.2.1.1	Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE	3770

A.16.3.2.1.2	Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE	3773

A.16.3.2.1.3	Inter-frequency RRC Re-establishment in FR1 for 1 Rx UE	3776

A.16.3.2.1.4	Inter-frequency RRC Re-establishment in FR1 for 2 Rx UE	3779

A.16.3.2.1.5	Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE without serving cell timing	3782

A.16.3.2.1.6	Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE without serving cell timing	3784

A.16.3.2.2	Random Access	3787

A.16.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	3787

A.16.3.2.2.2	4-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	3791

A.16.3.2.2.3	4-step RA type non-contention based random access test in FR1 for NR standalone for 1 Rx UE	3794

A.16.3.2.2.4	4-step RA type non-contention based random access test in FR1 for NR standalone for 2 Rx UE	3797

A.16.3.2.2.5	2-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	3800

A.16.3.2.2.6	2-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	3803

A.16.3.2.2.7	2-step RA type non-contention based test in FR1 for NR standalone for 1 RX UE	3806

A.16.3.2.2.8	2-step RA type non-contention based test in FR1 for NR standalone for 2 RX UE	3809

A.16.3.2.3	SA: RRC Connection Release with Redirection	3811

A.16.3.2.3.1	Redirection from NR in FR1 to NR in FR1 for 1 Rx UE	3811

A.16.3.2.3.2	Redirection from NR in FR1 to NR in FR1 for 2 Rx UE	3814

A.16.3.2.3.3	Redirection from NR in FR1 to E-UTRAN for 1 Rx UE	3816

A.16.3.2.3.4	Redirection from NR in FR1 to E-UTRAN for 2 Rx UE	3819

A.16.4	Timing for RedCap	3823

A.16.4.1	UE transmit timing	3823

A.16.4.1.1	NR UE Transmit Timing Test for FR1 for 1 Rx RedCap UE	3823

A.16.4.1.1.1	Test Purpose and environment	3823

A.16.4.1.1.2	Test requirements	3825

A.16.4.1.2	NR UE Transmit Timing Test for FR1 for 2 Rx RedCap UE	3825

A.16.4.1.2.1	Test Purpose and environment	3825

A.16.4.1.2.2	Test requirements	3827

A.16.4.2	Void	3827

A.16.4.3	Timing advance	3827

A.16.4.3.1	SA FR1 timing advance adjustment accuracy for 1 Rx UE	3827

A.16.4.3.1.1	Test Purpose and Environment	3827

A.16.4.3.1.2	Test Parameters	3827

A.16.4.3.1.3	Test Requirements	3830

A.16.4.3.2	SA FR1 timing advance adjustment accuracy for 2 Rx UE	3830

A.16.4.3.2.1	Test Purpose and Environment	3830

A.16.4.3.2.2	Test Parameters	3830

A.16.4.3.2.3	Test Requirements	3832

A.16.5	Signalling characteristics for RedCap	3833

A.16.5.1	Radio link Monitoring	3833

A.16.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	3833

A.16.5.1.1.1	Test Purpose and Environment	3833

A.16.5.1.1.2	Test Requirements	3835

A.16.5.1.2	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	3836

A.16.5.1.2.1	Test Purpose and Environment	3836

A.16.5.1.2.2	Test Requirements	3838

A.16.5.1.3	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	3839

A.16.5.1.3.1	Test Purpose and Environment	3839

A.16.5.1.3.2	Test Requirements	3841

A.16.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	3842

A.16.5.1.4.1	Test Purpose and Environment	3842

A.16.5.1.4.2	Test Requirements	3844

A.16.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	3845

A.16.5.1.5.1	Test Purpose and Environment	3845

A.16.5.1.5.2	Test Requirements	3847

A.16.5.1.6	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	3847

A.16.5.1.6.1	Test Purpose and Environment	3847

A.16.5.1.6.2	Test Requirements	3850

A.16.5.1.7	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	3850

A.16.5.1.7.1	Test Purpose and Environment	3850

A.16.5.1.7.2	Test Requirements	3853

A.16.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	3854

A.16.5.1.8.1	Test Purpose and Environment	3854

A.16.5.1.8.2	Test Requirements	3857

A.16.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	3857

A.16.5.1.9.1	Test Purpose and Environment	3857

A.16.5.1.9.2	Test Requirements	3860

A.16.5.1.10	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	3860

A.16.5.1.10.1	Test Purpose and Environment	3860

A.16.5.1.10.2	Test Requirements	3863

A.16.5.1.11	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	3863

A.16.5.1.11.1	Test Purpose and Environment	3863

A.16.5.1.11.2	Test Requirements	3866

A.16.5.1.12	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	3866

A.16.5.1.12.1	Test Purpose and Environment	3866

A.16.5.1.12.2	Test Requirements	3869

A.16.5.1.13	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE	3870

A.16.5.1.13.1	Test Purpose and Environment	3870

A.16.5.1.13.2	Test Requirements	3872

A.16.5.1.14	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	3873

A.16.5.1.14.1	Test Purpose and Environment	3873

A.16.5.1.14.2	Test Requirements	3875

A.16.5.1.15	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE	3876

A.16.5.1.15.1	Test Purpose and Environment	3876

A.16.5.1.15.2	Test Requirements	3879

A.16.5.1.16	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	3879

A.16.5.1.16.1	Test Purpose and Environment	3879

A.16.5.1.16.2	Test Requirements	3882

A.16.5.2	Beam Failure Detection and Link recovery procedures	3882

A.16.5.2.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 1 Rx UE	3882

A.16.5.2.1.1	Test Purpose and Environment	3882

A.16.5.2.1.2	Test Requirements	3885

A.16.5.2.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE	3886

A.16.5.2.2.1	Test Purpose and Environment	3886

A.16.5.2.2.2	Test Requirements	3889

A.16.5.2.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode for 1 Rx UE	3889

A.16.5.2.3.1	Test Purpose and Environment	3889

A.16.5.2.3.2	Test Requirements	3892

A.16.5.2.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode for 2 Rx UE	3893

A.16.5.2.4.1	Test Purpose and Environment	3893

A.16.5.2.4.2	Test Requirements	3896

A.16.5.2.5	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 1 Rx UE	3896

A.16.5.2.5.1	Test Purpose and Environment	3896

A.16.5.2.5.2	Test Requirements	3899

A.16.5.2.6	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 2 Rx UE	3900

A.16.5.2.6.1	Test Purpose and Environment	3900

A.16.5.2.6.2	Test Requirements	3903

A.16.5.2.7	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 1 Rx UE	3903

A.16.5.2.7.1	Test Purpose and Environment	3903

A.16.5.2.7.2	Test Requirements	3906

A.16.5.2.8	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 2 Rx UE	3907

A.16.5.2.8.1	Test Purpose and Environment	3907

A.16.5.2.8.2	Test Requirements	3910

A.16.5.3	Active BWP switch	3910

A.16.5.3.1	DCI-based and Timer-based Active BWP Switch	3910

A.16.5.3.1.1	NR FR1 DL active BWP switch with non-DRX in SA for 1 Rx UE	3910

A.16.5.3.1.1.1	Test Purpose and Environment	3910

A.16.5.3.1.1.2	Test Requirements	3913

A.16.5.3.1.2	NR FR1 DL active BWP switch with non-DRX in SA for 2 Rx UE	3913

A.16.5.3.1.2.1	Test Purpose and Environment	3913

A.16.5.3.1.2.2	Test Requirements	3915

A.16.5.3.2	RRC-based Active BWP Switch	3916

A.16.5.3.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA for 1 Rx UE	3916

A.16.5.3.2.1.1	Test Purpose and Environment	3916

A.16.5.3.2.1.2	Test Requirements	3918

A.16.5.3.2.2	NR FR1 DL active BWP switch of Cell with non-DRX in SA for 2 Rx UE	3918

A.16.5.3.2.2.1	Test Purpose and Environment	3918

A.16.5.3.2.2.2	Test Requirements	3920

A.16.5.4	UE specific CBW change	3921

A.16.5.4.1	UE specific CBW change on PCell in FR1 in non-DRX for 1 Rx UE	3921

A.16.5.4.1.1	Test Purpose and Environment	3921

A.16.5.4.1.2	Test Requirements	3923

A.16.5.4.2	UE specific CBW change on PCell in FR1 in non-DRX for 2 Rx UE	3923

A.16.5.4.2.1	Test Purpose and Environment	3923

A.16.5.4.2.2	Test Requirements	3926

A.16.6	Measurement procedure for RedCap	3926

A.16.6.1	Intra-frequency Measurements	3926

A.16.6.1.1	SA event triggered reporting tests without gap under non-DRX for 1 Rx UE	3926

A.16.6.1.1.1	Test purpose and Environment	3926

A.16.6.1.1.2	Test parameters	3926

A.16.6.1.1.3	Test Requirements	3928

A.16.6.1.2	SA event triggered reporting tests without gap under non-DRX for 2 Rx UE	3928

A.16.6.1.2.1	Test purpose and Environment	3928

A.16.6.1.2.2	Test parameters	3928

A.16.6.1.2.3	Test Requirements	3930

A.16.6.1.3	SA event triggered reporting tests without gap under DRX for 1 Rx UE	3930

A.16.6.1.3.1	Test purpose and Environment	3930

A.16.6.1.3.2	Test parameters	3930

A.16.6.1.3.3	Test Requirements	3932

A.16.6.1.4	SA event triggered reporting tests without gap under DRX for 2 Rx UE	3933

A.16.6.1.4.1	Test purpose and Environment	3933

A.16.6.1.4.2	Test parameters	3933

A.16.6.1.4.3	Test Requirements	3935

A.16.6.1.5	SA event triggered reporting tests with per-UE gaps under non-DRX for 1 Rx UE	3935

A.16.6.1.5.1	Test purpose and Environment	3935

A.16.6.1.5.2	Test parameters	3935

A.16.6.1.5.3	Test Requirements	3937

A.16.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX for 2 Rx UE	3937

A.16.6.1.6.1	Test purpose and Environment	3937

A.16.6.1.6.2	Test parameters	3937

A.16.6.1.6.3	Test Requirements	3939

A.16.6.1.7	SA event triggered reporting tests with per-UE gaps under DRX for 1 Rx UE	3940

A.16.6.1.7.1	Test purpose and Environment	3940

A.16.6.1.7.2	Test parameters	3940

A.16.6.1.7.3	Test Requirements	3942

A.16.6.1.8	SA event triggered reporting tests with per-UE gaps under DRX for 2 Rx UE	3942

A.16.6.1.8.1	Test purpose and Environment	3942

A.16.6.1.8.2	Test parameters	3942

A.16.6.1.8.3	Test Requirements	3944

A.16.6.1.9	SA event triggered reporting tests without gap under non-DRX with SSB index reading for 1 Rx UE	3944

A.16.6.1.9.1	Test purpose and Environment	3944

A.16.6.1.9.2	Test parameters	3945

A.16.6.1.9.3	Test Requirements	3946

A.16.6.1.10	SA event triggered reporting tests without gap under non-DRX with SSB index reading for 2 Rx UE	3946

A.16.6.1.10.1	Test purpose and Environment	3946

A.16.6.1.10.2	Test parameters	3946

A.16.6.1.10.3	Test Requirements	3948

A.16.6.1.11	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 1 Rx UE	3948

A.16.6.1.11.1	Test purpose and Environment	3948

A.16.6.1.11.2	Test parameters	3948

A.16.6.1.11.3	Test Requirements	3950

A.16.6.1.12	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 2 Rx UE	3950

A.16.6.1.12.1	Test purpose and Environment	3950

A.16.6.1.12.2	Test parameters	3950

A.16.6.1.12.3	Test Requirements	3951

A.16.6.2	Inter-frequency Measurements	3952

A.16.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 1 Rx UE	3952

A.16.6.2.1.1	Test Purpose and Environment	3952

A.16.6.2.1.2	Test Requirements	3954

A.16.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 2 Rx UE	3955

A.16.6.2.2.1	Test Purpose and Environment	3955

A.16.6.2.2.2	Test Requirements	3957

A.16.6.2.3	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used for 1 Rx UE	3957

A.16.6.2.3.1	Test Purpose and Environment	3958

A.16.6.2.3.2	Test Requirements	3960

A.16.6.2.4	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used for 2 Rx UE	3960

A.16.6.2.4.1	Test Purpose and Environment	3960

A.16.6.2.4.2	Test Requirements	3962

A.16.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 1 Rx UE	3962

A.16.6.2.5.1	Test Purpose and Environment	3963

A.16.6.2.5.2	Test Requirements	3965

A.16.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 2 Rx UE	3965

A.16.6.2.6.1	Test Purpose and Environment	3965

A.16.6.2.6.2	Test Requirements	3967

A.16.6.2.7	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 1 Rx UE	3967

A.16.6.2.7.1	Test Purpose and Environment	3967

A.16.6.2.7.2	Test Requirements	3970

A.16.6.2.8	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 2 Rx UE	3970

A.16.6.2.8.1	Test Purpose and Environment	3970

A.16.6.2.8.2	Test Requirements	3972

A.16.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern for 1 Rx UE	3973

A.16.6.2.9.1	Test Purpose and Environment	3973

A.16.6.2.9.2	Test Requirements	3975

A.16.6.2.10	SA event triggered reporting tests with additional mandatory gap pattern for 2 Rx UE	3975

A.16.6.2.10.1	Test Purpose and Environment	3975

A.16.6.2.10.2	Test Requirements	3977

A.16.6.2.11	SA event triggered reporting tests for FR1 when DRX is used for 1 Rx UE	3978

A.16.6.2.11.1	Test Purpose and Environment	3978

A.16.6.2.11.2	Test Requirements	3980

A.16.6.2.12	SA event triggered reporting tests for FR1 when DRX is used for 2 Rx UE	3980

A.16.6.2.12.1	Test Purpose and Environment	3980

A.16.6.2.12.2	Test Requirements	3983

A.16.6.3	Inter-RAT Measurements	3983

A.16.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 for 1 Rx UE	3983

A.16.6.3.1.1	Test purpose and Environment	3983

A.16.6.3.1.2	Test Requirements	3986

A.16.6.3.2	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 for 2 Rx UE	3987

A.16.6.3.2.1	Test purpose and Environment	3987

A.16.6.3.2.2	Test Requirements	3990

A.16.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for 1 Rx UE	3990

A.16.6.3.3.1	Test purpose and Environment	3990

A.16.6.3.3.2	Test Requirements	3994

A.16.6.3.4	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for 2 Rx UE	3994

A.16.6.3.4.1	Test purpose and Environment	3994

A.16.6.3.4.2	Test Requirements	3997

A.16.6.4	L1-RSRP measurement for beam reporting	3998

A.16.6.4.1	SSB based L1-RSRP measurement when DRX is not used for 1 Rx UE	3998

A.16.6.4.1.1	Test Purpose and Environment	3998

A.16.6.4.1.2	Test parameters	3998

A.16.6.4.1.3	Test Requirements	3999

A.16.6.4.2	SSB based L1-RSRP measurement when DRX is not used for 2 Rx UE	4000

A.16.6.4.2.1	Test Purpose and Environment	4000

A.16.6.4.2.2	Test parameters	4000

A.16.6.4.2.3	Test Requirements	4001

A.16.6.4.3	SSB based L1-RSRP measurement when DRX is used for 1 Rx UE	4001

A.16.6.4.3.1	Test Purpose and Environment	4001

A.16.6.4.3.2	Test parameters	4002

A.16.6.4.3.3	Test Requirements	4003

A.16.6.4.4	SSB based L1-RSRP measurement when DRX is used for 2 Rx UE	4003

A.16.6.4.4.1	Test Purpose and Environment	4003

A.16.6.4.4.2	Test parameters	4004

A.16.6.4.4.3	Test Requirements	4005

A.16.6.4.5	CSI-RS based L1-RSRP measurement when DRX is not used for 1 Rx UE	4005

A.16.6.4.5.1	Test Purpose and Environment	4005

A.16.6.4.5.2	Test parameters	4006

A.16.6.4.5.3	Test Requirements	4007

A.16.6.4.6	CSI-RS based L1-RSRP measurement when DRX is not used for 2 Rx UE	4007

A.16.6.4.6.1	Test Purpose and Environment	4007

A.16.6.4.6.2	Test parameters	4008

A.16.6.4.6.3	Test Requirements	4009

A.16.6.4.7	CSI-RS based L1-RSRP measurement when DRX is used for 1 Rx UE	4009

A.16.6.4.7.1	Test Purpose and Environment	4009

A.16.6.4.7.2	Test parameters	4010

A.16.6.4.7.3	Test Requirements	4011

A.16.6.4.8	CSI-RS based L1-RSRP measurement when DRX is used for 2 Rx UE	4011

A.16.6.4.8.1	Test Purpose and Environment	4011

A.16.6.4.8.2	Test parameters	4012

A.16.6.4.8.3	Test Requirements	4013

A.16.6.5	NR measurements with autonomous gaps	4014

A.16.6.5.1	SA intra-frequency CGI identification of NR neighbor cell in FR1 for 1 Rx UE	4014

A.16.6.5.1.1	Test Purpose and Environment	4014

A.16.6.5.1.2	Test Parameters	4014

A.16.6.5.1.3	Test Requirements	4016

A.16.6.5.2	SA intra-frequency CGI identification of NR neighbor cell in FR1 for 2 Rx UE	4016

A.16.6.5.2.1	Test Purpose and Environment	4016

A.16.6.5.2.2	Test Parameters	4016

A.16.6.5.2.3	Test Requirements	4018

A.16.6.5.3	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 1 Rx UE	4018

A.16.6.5.3.1	Test Purpose and Environment	4018

A.16.6.5.3.2	Test Requirements	4021

A.16.6.5.4	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 2 Rx UE	4021

A.16.6.5.4.1	Test Purpose and Environment	4021

A.16.6.5.4.2	Test Requirements	4024

A.16.6.6	RSTD Measurements	4024

A.16.6.6.1	NR RSTD measurement reporting delay test case for RedCap UE without FH in FR1 SA	4024

A.16.6.6.1.1	Test Purpose and Environment	4024

A.16.6.6.1.2	Test Requirements	4029

A.16.6.6.2	NR RSTD measurement reporting delay test case with PRS frequency hopping	4029

A.16.6.6.2.1	Test Purpose and Environment	4029

A.16.6.6.2.2	Test Requirements	4033

A.16.6.7	UE Rx-Tx Measurements	4034

A.16.6.7.1	UE Rx-Tx measurement reporting delay test case for single positioning frequency layer in FR1 SA for RedCap UE without RX FH in RRC_CONNECTED mode	4034

A.16.6.7.1.1	Test purpose and environment	4034

A.16.6.7.1.2	Test requirements	4038

A.16.6.7.2	UE Rx-Tx time difference measurement with Rx FH for single positioning frequency layer in FR1 SA in RRC_CONNECTED state	4038

A.16.6.7.2.1	Test purpose and environment	4038

A.16.6.7.2.2	Test requirements	4042

A.16.6.8	PRS-RSRP measurements	4042

A.16.6.8.1	PRS-RSRP measurement delay test case for single positioning frequency layer	4042

A.16.6.8.1.1	Test purpose and Environment	4042

A.16.6.8.1.2	Test Requirements	4046

A.16.6.8.2	PRS-RSRP measurement delay with FH in RRC_CONNECTED state in FR1	4046

A.16.6.8.2.1	Test purpose and Environment	4046

A.16.6.8.2.2	Test Requirements	4050

A.16.6.9	PRS-RSRPP Measurements	4050

A.16.6.9.1	PRS-RSRPP measurement delay without FH in RRC_CONNECTED state in FR1	4050

A.16.6.9.1.1	Test purpose and Environment	4050

A.16.6.9.1.2	Test Requirements	4052

A.16.6.9.2	PRS-RSRPP measurement with Rx FH reporting delay test case for single positioning frequency layer in FR1 SA in RRC_CONNECTED state	4053

A.16.6.9.2.1	Test purpose and Environment	4053

A.16.6.9.2.2	Test Requirements	4055

A.16.7	Measurement Performance requirements for RedCap	4055

A.16.7.1	SS-RSRP	4055

A.16.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	4055

A.16.7.1.1.1	Test Purpose and Environment	4055

A.16.7.1.1.2	Test parameters	4055

A.16.7.1.1.3	Test Requirements	4059

A.16.7.1.2	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 2 RX UE	4059

A.16.7.1.2.1	Test Purpose and Environment	4059

A.16.7.1.2.2	Test parameters	4059

A.16.7.1.2.3	Test Requirements	4063

A.16.7.1.3	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	4063

A.16.7.1.3.1	Test Purpose and Environment	4063

A.16.7.1.3.2	Test parameters	4063

A.16.7.1.3.3	Test Requirements	4066

A.16.7.1.4	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	4066

A.16.7.1.4.1	Test Purpose and Environment	4066

A.16.7.1.4.2	Test parameters	4066

A.16.7.1.4.3	Test Requirements	4069

A.16.7.2	SS-RSRQ	4069

A.16.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	4069

A.16.7.2.1.1	Test Purpose and Environment	4069

A.16.7.2.1.2	Test Parameters	4069

A.16.7.2.1.3	Test Requirements	4073

A.16.7.2.2	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	4073

A.16.7.2.2.1	Test Purpose and Environment	4073

A.16.7.2.2.2	Test Parameters	4073

A.16.7.2.2.3	Test Requirements	4076

A.16.7.2.3	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	4076

A.16.7.2.3.1	Test Purpose and Environment	4076

A.16.7.2.3.2	Test parameters	4077

A.16.7.2.3.3	Test Requirements	4080

A.16.7.2.4	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	4080

A.16.7.2.4.1	Test Purpose and Environment	4080

A.16.7.2.4.2	Test parameters	4080

A.16.7.2.4.3	Test Requirements	4084

A.16.7.3	SS-SINR	4084

A.16.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	4084

A.16.7.3.1.1	Test Purpose and Environment	4084

A.16.7.3.1.2	Test parameters	4084

A.16.7.3.1.3	Test Requirements	4087

A.16.7.3.2	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	4087

A.16.7.3.2.1	Test Purpose and Environment	4087

A.16.7.3.2.2	Test parameters	4087

A.16.7.3.3	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	4090

A.16.7.3.3.1	Test Purpose and Environment	4090

A.16.7.3.3.2	Test parameters	4090

A.16.7.3.3.3	Test Requirements	4093

A.16.7.3.4	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	4093

A.16.7.3.4.1	Test Purpose and Environment	4093

A.16.7.3.4.2	Test parameters	4093

A.16.7.3.4.3	Test Requirements	4096

A.16.7.4	L1-RSRP measurement for beam reporting	4097

A.16.7.4.1	SSB based L1-RSRP measurement for 1 Rx UE	4097

A.16.7.4.1.1	Test Purpose and Environment	4097

A.16.7.4.1.2	Test parameters	4097

A.16.7.4.1.3	Test Requirements	4099

A.16.7.4.2	SSB based L1-RSRP measurement for 2 Rx UE	4099

A.16.7.4.2.1	Test Purpose and Environment	4099

A.16.7.4.2.2	Test parameters	4100

A.16.7.4.2.3	Test Requirements	4100

A.16.7.4.3	CSI-RS based L1-RSRP measurement on resource set with repetition off for 1 Rx UE	4100

A.16.7.4.3.1	Test Purpose and Environment	4100

A.16.7.4.3.2	Test parameters	4100

A.16.7.4.3.3	Test Requirements	4103

A.16.7.4.4	CSI-RS based L1-RSRP measurement on resource set with repetition off for 2 Rx UE	4103

A.16.7.4.4.1	Test Purpose and Environment	4103

A.16.7.4.4.2	Test parameters	4103

A.16.7.4.4.3	Test Requirements	4103

A.16.7.5	E-UTRAN RSRP	4103

A.16.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell for 1 Rx UE	4103

A.16.7.5.1.1	Test Purpose and Environment	4103

A.16.7.5.1.2	Test parameters	4103

A.16.7.5.1.3	Test Requirements	4107

A.16.7.5.2	SA: inter-RAT measurement accuracy with FR1 serving cell for 2 Rx UE	4107

A.16.7.5.2.1	Test Purpose and Environment	4107

A.16.7.5.2.2	Test parameters	4107

A.16.7.5.2.3	Test Requirements	4110

A.16.7.6	E-UTRAN RSRQ	4110

A.16.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell for 1 Rx UE	4110

A.16.7.6.1.1	Test Purpose and Environment	4110

A.16.7.6.1.2	Test parameters	4110

A.16.7.6.1.3	Test Requirements	4114

A.16.7.6.2	SA: inter-RAT measurement accuracy with FR1 serving cell for 2 Rx UE	4114

A.16.7.6.2.1	Test Purpose and Environment	4114

A.16.7.6.2.2	Test parameters	4114

A.16.7.6.2.3	Test Requirements	4117

A.16.7.7	RSTD measurements	4117

A.16.7.7.1	RSTD measurement accuracy test case for RedCap UE without FH	4117

A.16.7.7.1.1	Test purpose and Environment	4117

A.16.7.7.1.2	Test Requirements	4119

A.16.7.8	UE Rx-Tx measurements	4123

A.16.7.8.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA for RedCap UE without RX FH in RRC_CONNECTED mode	4123

A.16.7.8.1.1	Test purpose and environment	4123

A.16.7.8.1.2	Test parameters	4124

A.16.7.8.1.3	Test requirements	4127

A.16.7.8.2	SA: UE Rx-Tx time difference measurement accuracy with Rx FH in RRC_CONNECTED state in FR1	4127

A.16.7.8.2.1 	Test purpose and Environment	4127

A.16.7.8.2.2	Test parameters	4128

A.16.7.8.2.3	Test requirements	4131

A.16.7.9	PRS-RSRP Measurements	4131

A.16.7.9.1	PRS-RSRP measurement accuracy without FH in RRC_CONNECTED state in FR1	4131

A.16.7.9.1.1	Test Purpose and Environment	4131

A.16.7.9.1.2	Test parameters	4131

A.16.7.9.1.3	Test Requirements	4135

A.16.7.9.2	PRS-RSRP measurement accuracy with FH in RRC_CONNECTED state in FR1	4136

A.16.7.9.2.1	Test Purpose and Environment	4136

A.16.7.9.2.2	Test parameters	4136

A.16.7.9.2.3	Test Requirements	4139

A.16.7.10	PRS-RSRPP measurements	4140

A.16.7.10.1	PRS-RSRPP measurement accuracy without FH in RRC_CONNECTED state in FR1	4140

A.16.7.10.1.1	Test Purpose and Environment	4140

A.16.7.10.1.2	Test parameters	4140

A.16.7.10.1.3	Test Requirements	4143

A.16.7.10.2	SA: PRS-RSRPP measurement accuracy with Rx FH in RRC_CONNECTED state in FR1	4143

A.16.7.10.2.1	Test purpose and Environment	4143

A.16.7.10.2.2	Test parameters	4143

A.16.7.10.2.3	Test requirements	4147

A.16.8	Measurement Procedure for RedCap in RRC_INACTIVE	4147

A.16.8.1	RSTD Measurements	4147

A.16.8.1.1	NR RSTD measurement reporting delay test case for for RedCap UE without FH in FR1 SA in RRC_INACTIVE state	4147

A.16.8.1.1.1	Test Purpose and Environment	4147

A.16.8.1.1.2	Test Requirements	4151

A.16.8.1.2	NR RSTD measurement reporting delay test case with PRS frequency hopping	4151

A.16.8.1.2.1	Test Purpose and Environment	4151

A.16.8.1.2.2	Test Requirements	4155

A.16.8.1.3	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_INACTIVE state when eDRX cycle > 10.24s for RedCap UE	4155

A.16.8.1.3.1	Test Purpose and Environment	4155

A.16.8.1.3.2	Test Requirements	4159

A.16.8.2	UE Rx-Tx Measurements	4159

A.16.8.2.1	UE Rx-Tx measurement reporting delay test case for single positioning frequency layer in FR1 SA for RedCap UE without RX FH in RRC_INACTIVE mode	4159

A.16.8.2.1.1	Test purpose and environment	4159

A.16.8.2.1.2	Test requirements	4163

A.16.8.2.2	UE Rx-Tx time difference measurement with Rx FH for single positioning frequency layer in FR1 SA in RRC_INACTIVE state	4163

A.16.8.2.2.1	Test purpose and environment	4163

A.16.8.2.2.2	Test requirements	4167

A.16.8.2.3.	UE Rx-Tx time difference measurement for single positioning frequency layer with eDRX > 10.24s in FR1 SA	4167

A.16.8.2.3.1	Test purpose and environment	4167

A.16.8.2.3.2	Test requirements	4171

A.16.8.3	PRS-RSRP Measurements	4171

A.16.8.3.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_INACTIVE	4171

A.16.8.3.1.1	Test purpose and Environment	4171

A.16.8.3.1.2	Test Requirements	4173

A.16.8.3.3	PRS-RSRP reporting delay test case in RRC_INACTIVE state in FR1 when eDRX cycle > 10.24s	4174

A.16.8.3.3.1	Test purpose and Environment	4174

A.16.8.3.3.2	Test Requirements	4176

A.16.8.4	PRS-RSRPP Measurements	4177

A.16.8.4.1	PRS-RSRPP measurement delay without FH in RRC_INACTIVE state in FR1	4177

A.16.8.4.1.1	Test purpose and Environment	4177

A.16.8.4.2	PRS-RSRPP measurement with Rx FH reporting delay test case for single positioning frequency layer in FR1 SA in RRC_INACTIVE state	4180

A.16.8.4.2.1	Test purpose and Environment	4180

A.16.8.4.2.2	Test Requirements	4182

A.16.9	Measurement Performance Requirements for RedCap in RRC_INACTIVE	4185

A.16.9.1	 RSTD Measurements	4185

A.16.9.1.1	RSTD measurement accuracy test case for RedCap UE without FH in FR1 in RRC_INACTIVE state	4185

A.16.9.1.1.1	Test purpose and Environment	4185

A.16.9.1.1.2	Test Requirements	4187

A.16.9.1.2	RSTD measurement accuracy test case for RedCap UE with FH in FR1 in RRC_INACTIVE state	4187

A.16.9.1.2.1	Test purpose and Environment	4187

A.16.9.1.2.2	Test Requirements	4189

A.16.9.2	UE Rx-Tx measurements	4189

A.16.9.2.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA for RedCap UE without RX FH in RRC_INACTIVE mode	4189

A.16.9.2.1.1	Test purpose and environment	4189

A.16.9.2.1.2	Test parameters	4190

A.16.9.2.1.3	Test requirements	4191

A.16.9.2.2	SA: UE Rx-Tx time difference measurement accuracy with Rx FH in RRC_INACTIVE state in FR1	4192

A.16.9.2.2.1	Test purpose and Environment	4192

A.16.9.2.2.2	Test parameters	4192

A.16.9.2.2.3	Test requirements	4195

A.16.9.3	PRS-RSRP Measurements	4195

A.16.9.3.1	PRS-RSRP measurement accuracy without FH in RRC_INACTIVE state in FR1	4195

A.16.9.3.1.1	Test Purpose and Environment	4195

A.16.9.3.1.2	Test parameters	4195

A.16.9.3.1.3	Test Requirements	4198

A.16.9.3.2	PRS-RSRP measurement accuracy with FH in RRC_INACTIVE state in FR1	4198

A.16.9.3.2.1	Test Purpose and Environment	4198

A.16.9.3.2.2	Test parameters	4198

A.16.9.3.2.3	Test Requirements	4201

A.16.9.4	PRS-RSRPP measurements	4201

A.16.9.4.1	PRS-RSRPP measurement accuracy without Rx FH in RRC_INACTIVE state in FR1	4201

A.16.9.4.1.1	Test purpose and Environment	4201

A.16.9.4.1.2	Test parameters	4201

A.16.9.4.1.3	Test requirements	4205

A.16.9.4.2	SA: PRS-RSRPP measurement accuracy with Rx FH in RRC_INACTIVE state in FR1	4205

A.16.9.4.2.1	Test purpose and Environment	4205

A.16.9.4.2.2	Test parameters	4205

A.16.9.4.2.3	Test requirements	4208

A.16.10	 Measurement procedure for RedCap in RRC_IDLE	4208

A.16.10.1	RSTD measurements	4208

A.16.10.1.1	NR RSTD measurement reporting delay test case for RedCap UE without FH in FR1 SA in RRC_IDLE state without eDRX	4208

A.16.10.1.1.1	Test Purpose and Environment	4208

A.16.10.1.1.2	Test Requirements	4212

A.16.10.1.2	NR RSTD measurement reporting delay test case for RedCap UE without RX FH in FR1 SA in RRC_IDLE state when eDRX > 10.24s	4212

A.16.10.1.2.1	Test Purpose and Environment	4212

A.16.10.1.2.2	Test Requirements	4216

A.16.10.2	PRS-RSRP Measurements	4216

A.16.10.2.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_IDLE	4216

A.16.10.2.1.1	Test purpose and Environment	4216

A.16.10.2.1.2	Test Requirements	4218

A.16.10.2.2	PRS-RSRP measurement without Rx FH reporting delay test case for single positioning frequency layer in FR1 SA in RRC_IDLE state with eDRX cycle > 10.24s	4219

A.16.10.2.2.1	Test purpose and Environment	4219

A.16.10.2.2.2	Test Requirements	4221

A.16.11	 Measurement Performance Requirements for RedCap in RRC_IDLE	4221

A.16.11.1	RSTD Measurements	4221

A.16.11.1.1	RSTD measurement accuracy test case for RedCap UE without FH in FR1 in RRC_IDLE state without eDRX	4221

A.16.11.1.1.1	Test purpose and Environment	4221

A.16.11.1.1.2	Test Requirements	4223

A.16.11.1.2	RSTD measurement accuracy test case for RedCap UE without FH in FR1 in RRC_IDLE state with eDRX > 10.24s	4224

A.16.11.1.2.1	Test purpose and Environment	4224

A.16.11.1.2.2	Test Requirements	4226

A.16.11.2	PRS-RSRP Measurements	4226

A.16.11.2.1	PRS-RSRP measurement accuracy test case for RedCap UE in FR1 in RRC_IDLE state	4226

A.16.11.2.1.1	Test Purpose and Environment	4226

A.16.11.2.1.2	Test parameters	4226

A.16.11.2.1.3	Test Requirements	4228

A.16.11.2.2	PRS-RSRP measurement without Rx FH accuracy test case for single positioning frequency layer in FR1 SA in RRC_IDLE state with eDRX cycle > 10.24s	4228

A.16.11.2.2.1	Test purpose and Environment	4228

A.16.11.2.2.2	Test Requirements	4230

A.17	NR standalone tests with one or more NR cells in FR2 for RedCap	4231

A.17.1	SA: RRC_IDLE state mobility for RedCap	4231

A.17.1.1	Cell re-selection to NR	4231

A.17.1.1.1	Cell reselection to FR2 intra-frequency NR case for 2 Rx	4231

A.17.1.1.1.1	Test Purpose and Environment	4231

A.17.1.1.1.2	Test Parameters	4231

A.17.1.1.1.3	Test Requirements	4233

A.17.1.1.2	Cell reselection to FR2 inter-frequency NR case	4233

A.17.1.1.2.1	Test Purpose and Environment	4233

A.17.1.1.2.2	Test Parameters	4233

A.17.1.1.2.3	Test Requirements	4235

A.17.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	4236

A.17.1.1.3.1	Test Purpose and Environment	4236

A.17.1.1.3.2	Test Parameters	4236

A.17.1.1.3.3	Test Requirements	4238

A.17.1.1.4	Cell reselection to FR2 inter-frequency NR case for UE fulfilling stationary mobility relaxed measurement criterion for 2 Rx UE	4238

A.17.1.1.4.1	Test Purpose and Environment	4238

A.17.1.1.4.2	Test Parameters	4238

A.17.1.1.4.3	Test Requirements	4240

A.17.2	SA: RRC_INACTIVE state mobility for RedCap	4241

A.17.2.1	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap	4241

A.17.2.1.1	TA validation for CG-SDT in FR2 for RedCap	4241

A.17.2.1.1.1	Test Purpose and Environment	4241

A.17.2.1.1.2	Test Requirements	4244

A.17.2.2	Cell Reselection for Positioning	4244

A.17.2.2.1	Cell reselection to FR2 intra-frequency NR case with RRC_INACTIVE eDRX and positioning SRS	4244

A.17.2.2.1.1	Test Purpose and Environment	4244

A.17.2.2.1.2	Test Parameters	4244

A.17.2.2.1.3	Test Requirements	4244

A.17.3	RRC_CONNECTED state mobility for RedCap	4244

A.17.3.1	Handover for RedCap	4244

A.17.3.1.1	Intra-frequency handover from FR2 to FR2; unknown target cell for 2 Rx	4244

A.17.3.1.1.1	Test Purpose and Environment	4244

A.17.3.1.1.2	Test Parameters	4244

A.17.3.1.1.3	Test Requirements	4246

A.17.3.1.2	Inter-frequency handover from FR2 to FR2; unknown target cell for 2 Rx	4246

A.17.3.1.2.1	Test Purpose and Environment	4246

A.17.3.1.2.2	Test Parameters	4246

A.17.3.1.2.3 Test Requirements	4248

A.17.3.2	RRC Connection Mobility Control for RedCap	4248

A.17.3.2.1	SA: RRC Re-establishment	4248

A.17.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	4248

A.17.3.2.1.1.1	Test Purpose and Environment	4248

A.17.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	4250

A.17.3.2.1.2.1	Test Purpose and Environment	4250

A.17.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	4252

A.17.3.2.1.3.1	Test Purpose and Environment	4252

A.17.3.2.1.3.2	Test Requirements	4254

A.17.3.2.2	Random Access	4254

A.17.3.2.2.1	4-step RA type contention based random access test in FR2 for NR Standalone	4254

A.17.3.2.2.1.1	Test Purpose and Environment	4254

A.17.3.2.2.1.2	Test Requirements	4256

A.17.3.2.2.2	4-step RA type non-contention based random access test in FR2 for NR Standalone	4258

A.17.3.2.2.2.1	Test Purpose and Environment	4258

A.17.3.2.2.2.2	Test Requirements	4259

A.17.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	4261

A.17.3.2.2.3.1	Test Purpose and Environment	4261

A.17.3.2.2.3.2	Test Requirements	4262

A.17.3.2.2.4	2-step RA type non-contention based random access test in FR2 for NR Standalone	4263

A.17.3.2.2.4.1	Test Purpose and Environment	4263

A.17.3.2.2.4.2	Test Requirements	4265

A.17.3.2.3	SA: RRC Connection Release with Redirection	4266

A.17.3.2.3.1	Redirection from NR in FR2 to NR in FR2	4266

A.17.3.2.3.1.1	Test Purpose and Environment	4266

A.17.3.2.3.1.2	Test Parameters	4266

A.17.3.2.3.1.3	Test Requirements	4268

A.17.4	Timing	4268

A.17.4.1	UE transmit timing	4268

A.17.4.1.1	NR UE Transmit Timing Test for FR2	4268

A.17.4.1.1.1	Test Purpose and environment	4268

A.17.4.1.1.2	Test requirements	4270

A.17.4.2	UE timer accuracy	4271

A.17.4.3	Timing advance	4271

A.17.4.3.1	SA FR2 timing advance adjustment accuracy	4271

A.17.4.3.1.1	Test Purpose and Environment	4271

A.17.4.3.1.2	Test Parameters	4271

A.17.4.3.1.3 Test Requirements	4274

A.17.5	Signaling characteristics for RedCap	4274

A.17.5.1	Radio link Monitoring for RedCap	4274

A.17.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	4274

A.17.5.1.1.1	Test Purpose and Environment	4274

A.17.5.1.1.2	Test Requirements	4277

A.17.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	4277

A.17.5.1.2.1	Test Purpose and Environment	4277

A.17.5.1.2.2	Test Requirements	4280

A.17.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	4280

A.17.5.1.3.1	Test Purpose and Environment	4280

A.17.5.1.3.2	Test Requirements	4283

A.17.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	4283

A.17.5.1.4.1	Test Purpose and Environment	4283

A.17.5.1.4.2	Test Requirements	4285

A.17.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	4286

A.17.5.1.5.1	Test Purpose and Environment	4286

A.17.5.1.5.2	Test Requirements	4288

A.17.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	4289

A.17.5.1.6.1	Test Purpose and Environment	4289

A.17.5.1.6.2	Test Requirements	4291

A.17.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	4292

A.17.5.1.7.1	Test Purpose and Environment	4292

A.17.5.1.7.2	Test Requirements	4294

A.17.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	4294

A.17.5.1.8.1	Test Purpose and Environment	4294

A.17.5.1.8.2	Test Requirements	4297

A.17.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	4298

A.17.5.1.9.1	Test Purpose and Environment	4298

A.17.5.1.9.2	Test Requirements	4299

A.17.5.2	Beam Failure Detection and Link recovery procedures	4300

A.17.5.2.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	4300

A.17.5.2.1.1	Test Purpose and Environment	4300

A.17.5.2.1.2	Test Requirements	4302

A.17.5.2.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	4303

A.17.5.2.2.1	Test Purpose and Environment	4303

A.17.5.2.2.2	Test Requirements	4306

A.17.5.2.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	4306

A.17.5.2.3.1	Test Purpose and Environment	4306

A.17.5.2.3.2	Test Requirements	4309

A.17.5.2.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	4309

A.17.5.2.4.1	Test Purpose and Environment	4309

A.17.5.2.4.2	Test Requirements	4312

A.17.5.2.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE	4312

A.17.5.2.5.1	Test Purpose and Environment	4312

A.17.5.2.5.2	Test Requirements	4315

A.17.5.3	Active BWP switch for RedCap	4316

A.17.5.3.1	DCI-based and Timer-based Active BWP Switch	4316

A.17.5.3.1.1	NR FR2 DL active BWP switch with non-DRX in SA	4316

A.17.5.3.1.1.1	Test Purpose and Environment	4316

A.17.5.3.1.1.2	Test Requirements	4318

A.17.5.3.2	RRC-based Active BWP Switch	4318

A.17.5.3.2.1	NR FR2 DL active BWP switch of PCell with non-DRX in SA	4318

A.17.5.3.2.1.1	Test Purpose and Environment	4318

A.17.5.3.2.1.2	Test Requirements	4321

A.17.5.4	Active TCI state switch delay	4321

A.17.5.4.1	MAC-CE based active TCI state switch	4321

A.17.5.4.1.1	NR PCell FR2 active TCI state switch for a known TCI state	4321

A.17.5.4.1.1.1	Test Purpose and Environment	4321

A.17.5.4.1.1.2	Test Requirements	4324

A.17.5.4.2	RRC based active TCI state switch	4324

A.17.5.4.2.1	NR PCell FR2 active TCI state switch for a known TCI state	4324

A.17.5.4.2.1.1	Test Purpose and Environment	4324

A.17.5.4.2.1.2	Test Requirements	4327

A.17.5.5	Uplink spatial relation switch delay	4327

A.17.5.5.1	MAC-CE based Spatial Relation switch	4327

A.17.5.5.1.1	 NR PCell FR2 spatial relation associated with known DL-RS	4327

A.17.5.5.1.1.1	Test Purpose and Environment	4327

A.17.5.5.1.1.2	Test Requirements	4329

A.17.5.5.2	RRC based spatial relation switch	4330

A.17.5.5.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	4330

A.17.5.5.2.1.2	Test Requirements	4332

A.17.5.6	UE specific CBW change	4332

A.17.5.6.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	4332

A.17.5.6.1.1	Test Purpose and Environment	4332

A.17.5.6.1.2	Test Requirements	4334

A.17.6	Measurement procedure for RedCap	4335

A.17.6.1	Intra-frequency Measurements	4335

A.17.6.1.1	SA event triggered reporting test without gap under non-DRX	4335

A.17.6.1.1.1	Test purpose and Environment	4335

A.17.6.1.1.2	Test Requirements	4337

A.17.6.1.2	SA event triggered reporting test without gap under DRX	4337

A.17.6.1.2.1	Test purpose and Environment	4337

A.7.6.1.2.2	Test Requirements	4338

A.17.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	4338

A.17.6.1.3.1	Test purpose and Environment	4338

A.17.6.1.3.2	Test Requirements	4341

A.17.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	4341

A.17.6.1.4.1	Test purpose and Environment	4341

A.17.6.1.4.2	Test Requirements	4343

A.17.6.2	Inter-frequency Measurements	4344

A.17.6.2.1	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2)	4344

A.17.6.2.1.1	Test Purpose and Environment	4344

A.17.6.2.1.2	Test Requirements	4346

A.17.6.2.2	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is used (PCell in FR2)	4346

A.17.6.2.2.1	Test Purpose and Environment	4346

A.17.6.2.2.2	Test Requirements	4348

A.17.6.2.3	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is not used (PCell in FR2)	4349

A.17.6.2.3.1	Test Purpose and Environment	4349

A.17.6.2.3.2	Test Requirements	4351

A.17.6.2.4	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is used (PCell in FR2) for 2 RX UE	4351

A.17.6.2.4.1	Test Purpose and Environment	4351

A.17.6.2.4.2	Test Requirements	4353

A.17.6.3	L1-RSRP measurement for beam reporting	4354

A.17.6.3.1	SSB based L1-RSRP measurement when DRX is not used	4354

A.17.6.3.1.1	Test Purpose and Environment	4354

A.17.6.3.1.2	Test parameters	4354

A.17.6.3.1.3	Test Requirements	4354

A.17.6.3.2	SSB based L1-RSRP measurement when DRX is used	4354

A.17.6.3.2.1	Test Purpose and Environment	4354

A.17.6.3.2.2	Test parameters	4355

A.17.6.3.2.3	Test Requirements	4356

A.17.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	4356

A.17.6.3.3.1	Test Purpose and Environment	4356

A.17.6.3.3.2	Test parameters	4356

A.17.6.3.3.3	Test Requirements	4358

A.17.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	4358

A.17.6.3.4.1	Test Purpose and Environment	4358

A.17.6.3.4.2	Test parameters	4359

A.7.6.3.3.3	Test Requirements	4360

A.17.6.4.1	SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2) for 2 RX UE	4360

A.17.6.4.1.1	Test Purpose and Environment	4360

A.17.6.4.1.2	Test Requirements	4363

A.17.6.5	RSTD measurements	4363

A.17.6.5.1	NR RSTD measurement reporting delay test case for RedCap UE without FH in FR2 SA	4363

A.17.6.5.1.1	Test Purpose and Environment	4363

A.17.6.5.1.2	Test Requirements	4370

A.17.6.5.2	NR RSTD measurement reporting delay test case with PRS frequency hopping	4370

A.17.6.5.2.1	Test Purpose and Environment	4370

A.17.6.5.2.2	Test Requirements	4375

A.17.6.6	UE Rx-Tx Measurements	4376

A.17.6.6.1	UE Rx-Tx measurement reporting delay for single positioning frequency layer in FR2 SA without RX FH in RRC_CONNECTED mode	4376

A.17.6.6.1.1	Test purpose and environment	4376

A.17.6.6.1.2	Test requirements	4380

A.17.6.6.2	UE Rx-Tx time difference measurement with Rx FH for single positioning frequency layer in FR2 SA in RRC_CONNECTED state	4380

A.17.6.6.2.1	Test purpose and environment	4380

A.17.6.6.2.2	Test requirements	4384

A.17.6.7	PRS-RSRP measurements	4384

A.17.6.7.1	PRS-RSRP measurement delay test case for RedCap positioning without Rx FH in RRC_CONNECTED state in FR2	4384

A.17.6.7.1.1	PRS-RSRP measurement delay test case for single positioning frequency layer	4384

A.17.6.7.1.1.1	Test Purpose and Environment	4384

A.17.6.7.1.1.2	Test Requirements	4388

A.17.6.7.1.2	PRS-RSRP measurement delay test case for dual positioning frequency layer	4388

A.17.6.7.1.2.1	Test Purpose and Environment	4388

A.17.6.7.1.2.2	Test Requirements	4392

A.17.6.7.2	PRS-RSRP measurement delay with FH in RRC_CONNECTED state in FR2	4392

A.17.6.7.2.1	Test Purpose and Environment	4392

A.17.6.7.2.2	Test Requirements	4396

A.17.6.8	PRS-RSRPP Measurements	4396

A.17.6.8.1	PRS-RSRPP measurement delay without FH in RRC_CONNECTED state in FR2	4396

A.17.6.8.1.1	Test Purpose and Environment	4396

A.17.6.8.1.2	Test Requirements	4399

A.17.6.8.2	PRS-RSRPP measurement with Rx FH reporting delay test case for single positioning frequency layer in FR2 SA in RRC_CONNECTED state	4399

A.17.6.8.2.1	Test Purpose and Environment	4399

A.17.6.8.2.2	Test Requirements	4401

A.17.7	Measurement Performance requirements	4402

A.17.7.1	SS-RSRP	4402

A.17.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	4402

A.17.7.1.1.1	Test Purpose and Environment	4402

A.17.7.1.1.2	Test parameters	4402

A.17.7.1.1.3	Test Requirements	4404

A.17.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	4404

A.17.7.1.2.1	Test Purpose and Environment	4404

A.17.7.1.2.2	Test parameters	4404

A.17.7.1.2.3	Test Requirements	4406

A.17.7.2	SS-RSRQ	4407

A.17.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	4407

A.17.7.2.1.1	Test Purpose and Environment	4407

A.17.7.2.1.2	Test Parameters	4407

A.17.7.2.1.3	Test Requirements	4409

A.17.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell for 2 Rx UE	4409

A.17.7.2.2.1	Test Purpose and Environment	4409

A.17.7.2.2.2	Test parameters	4409

A.17.7.2.2.3	Test Requirements	4411

A.17.7.2.3	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	4411

A.17.7.3	L1-RSRP measurement for beam reporting	4411

A.17.7.3.1	SSB based L1-RSRP measurement	4411

A.17.7.3.1.1	Test Purpose and Environment	4411

A.17.7.3.1.2	Test parameters	4411

A.17.7.3.1.3	Test Requirements	4412

A.17.7.3.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	4412

A.17.7.3.2.1	Test Purpose and Environment	4412

A.17.7.3.2.2	Test parameters	4412

A.17.7.3.2.3	Test Requirements	4412

A.17.7.4	SS-SINR	4413

A.17.7.4	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell for 2Rx UE	4413

A.17.7.4.1.1	Test Purpose and Environment	4413

A.17.7.4.1.2	Test parameters	4413

A.17.7.4.1.3	Test Requirements	4415

A.17.7.5	RSTD measurements	4415

A.17.7.5.1	RSTD measurement accuracy test case for RedCap UE without FH	4415

A.17.7.5.1.1	Test purpose and Environment	4415

A.17.7.5.1.2	Test Requirements	4417

A.17.7.6	UE Rx-Tx Measurements	4419

A.17.7.6.1	UE Rx-Tx measurement accuracy for single positioning frequency layer in FR2 SA without RX FH in RRC_CONNECTED mode	4419

A.17.7.6.1.1	Test purpose and environment	4419

A.17.7.6.1.2	Test parameters	4420

A.17.7.6.1.3	Test requirements	4423

A.17.7.6.2	SA: UE Rx-Tx time difference measurement accuracy with Rx FH in RRC_CONNECTED state in FR2	4423

A.17.7.6.2.1	Test purpose and Environment	4423

A.17.7.6.2.2	Test parameters	4424

A.17.7.6.2.3	Test requirements	4427

A.17.7.7	PRS-RSRP Measurements	4427

A.17.7.7.1	PRS-RSRP measurement accuracy without FH in RRC_CONNECTED state in FR2	4427

A.17.7.7.1.1	Test Purpose and Environment	4427

A.17.7.7.1.2	Test parameters	4427

A.17.7.7.1.3	Test Requirements	4430

A.17.7.7.2	PRS-RSRP measurement accuracy with FH in RRC_CONNECTED state in FR2	4430

A.17.7.7.2.1	Test Purpose and Environment	4430

A.17.7.7.2.2	Test parameters	4431

A.17.7.7.2.3	Test Requirements	4433

A.17.7.8	PRS-RSRPP Measurements	4433

A.17.7.8.1	PRS-RSRPP measurement accuracy without FH in RRC_CONNECTED state in FR2	4433

A.17.7.8.1.1	Test Purpose and Environment	4433

A.17.7.8.1.2	Test parameters	4434

A.17.7.8.1.3	Test Requirements	4436

A.17.7.8.2	SA: PRS-RSRPP measurement accuracy with Rx FH in RRC_CONNECTED state in FR2	4436

A.17.7.8.2.1	Test purpose and Environment	4436

A.17.7.8.2.2	Test parameters	4437

A.17.7.8.2.3	Test requirements	4439

A.17.8	Measurement Procedure for RedCap in RRC_INACTIVE	4440

A.17.8.1	RSTD Measurements	4440

A.17.8.1.1	NR RSTD measurement reporting delay test case for RedCap UE without FH in FR2 SA in RRC_INACTIVE state	4440

A.17.8.1.1.1	Test Purpose and Environment	4440

A.17.8.1.1.2	Test Requirements	4443

A.17.8.1.2	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_INACTIVE state	4443

A.17.8.1.2.1	Test Purpose and Environment	4443

A.17.8.1.2.2	Test Requirements	4446

A.17.8.1.3	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_INACTIVE state with eDRX > 10.24s	4446

A.17.8.1.3.1	Test purpose and environment	4446

A.17.8.1.3.2	Test requirements	4446

A.17.8.2	UE Rx-Tx Measurements	4447

A.17.8.2.1	UE Rx-Tx measurement reporting delay for single positioning frequency layer in FR2 SA without RX FH in RRC_INACTIVE mode	4447

A.17.8.2.1.1	Test purpose and environment	4447

A.17.8.2.1.2	Test requirements	4450

A.17.8.2.2	UE Rx-Tx time difference measurement with Rx FH for single positioning frequency layer in FR2 SA in RRC_INACTIVE state	4450

A.17.8.2.2.1	Test purpose and environment	4450

A.17.8.2.2.2	Test requirements	4454

A.17.8.2.3	UE Rx-Tx time difference measurements for single positioning frequency layer with eDRX > 10.24s in FR2 SA	4454

A.17.8.2.3.1	Test purpose and environment	4454

A.17.8.2.3.2	Test requirements	4454

A.17.8.3	PRS-RSRP Measurements	4455

A.17.8.3.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_INACTIVE	4455

A.17.8.3.1.1	Test Purpose and Environment	4455

A.17.8.3.1.2	Test Requirements	4459

A.17.8.3.2.2	Test Requirements	4463

A.17.8.3.3	PRS-RSRP reporting delay in RRC_INACTIVE with eDRX	4463

A.17.8.3.3.1	Test Purpose and Environment	4463

A.17.8.3.3.2	Test Requirements	4467

A.17.8.4	PRS-RSRPP Measurements	4467

A.17.8.4.1	PRS-RSRPP measurement delay without FH in RRC_INACTIVE state in FR2	4467

A.17.8.4.1.1	Test Purpose and Environment	4467

A.17.8.4.2	PRS-RSRPP measurement with Rx FH reporting delay test case for single positioning frequency layer in FR2 SA in RRC_INACTIVE state	4470

A.17.8.4.2.1	Test Purpose and Environment	4470

A.17.8.4.2.2	Test Requirements	4472

A.17.8.4.3	PRS-RSPP reporting delay in RRC_INACTIVE state with eDRX > 10.24s	4472

A.17.8.4.3.1	Test purpose and environment	4472

A.17.8.4.3.2	Test requirements	4472

A.17.9	Measurement Performance Requirements for RedCap in RRC_INACTIVE	4473

A.17.9.1	RSTD Measurements	4473

A.17.9.1.1	RSTD measurement accuracy test case for RedCap UE without FH in FR2 in RRC_INACTIVE state	4473

A.17.9.1.1.1	Test purpose and Environment	4473

A.17.9.1.1.2	Test Requirements	4475

A.17.9.1.2	RSTD measurement accuracy test case for RedCap UE with FH in FR2 in RRC_INACTIVE state	4475

A.17.9.1.2.1	Test purpose and Environment	4475

A.17.9.1.2.2	Test Requirements	4477

A.17.9.2	UE Rx-Tx Measurements	4477

A.17.9.2.1	UE Rx-Tx measurement accuracy for single positioning frequency layer in FR2 SA without RX FH in RRC_INACTIVE mode	4477

A.17.9.2.1.1	Test purpose and environment	4477

A.17.9.2.1.2	Test parameters	4478

A.17.9.2.1.3	Test requirements	4481

A.17.9.2.2	SA: UE Rx-Tx time difference measurement accuracy with Rx FH in RRC_INACTIVE state in FR2	4481

A.17.9.2.2.1	Test purpose and Environment	4481

A.17.9.2.2.2	Test parameters	4481

A.17.9.2.2.3	Test requirements	4484

A.17.9.3	PRS-RSRP Measurements	4484

A.17.9.3.2	PRS-RSRP measurement accuracy with FH in RRC_INACTIVE state in FR2	4486

A.17.9.3.2.1	Test Purpose and Environment	4486

A.17.9.3.2.2	Test parameters	4487

A.17.9.3.2.3	Test Requirements	4488

A.17.9.4	PRS-RSRPP Measurements	4488

A.17.9.4.1	SA: PRS-RSRPP measurement accuracy with Rx FH in RRC_INACTIVE state in FR2	4488

A.17.9.4.1.1	Test Purpose and Environment	4488

A.17.9.4.1.2	Test parameters	4489

A.17.9.4.1.3	Test Requirements	4491

A.17.9.4.2	SA: PRS-RSRPP measurement accuracy with Rx FH in RRC_INACTIVE state in FR2	4491

A.17.9.4.2.1	Test Purpose and Environment	4491

A.17.9.4.2.2	Test parameters	4492

A.17.9.4.2.3	Test Requirements	4494

A.17.10	Measurement Procedure for RedCap in RRC_IDLE	4495

A.17.10.1	RSTD Measurements	4495

A.17.10.1.1	NR RSTD measurement reporting delay test case for RedCap UE without FH in FR2 SA in RRC_IDLE state without eDRX	4495

A.17.10.1.1.1	Test Purpose and Environment	4495

A.17.10.1.1.2	Test Requirements	4498

A.17.10.1.2	NR RSTD without FH measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_IDLE state with eDRX > 10.24s	4498

A.17.10.1.2.1	Test purpose and environment	4498

A.17.10.1.2.2	Test requirements	4498

A.17.10.2	PRS-RSRP Measurements	4499

A.17.10.2.1	PRS-RSRP measurement delay test case for single positioning frequency layer in RRC_IDLE	4499

A.17.10.2.1.1	Test Purpose and Environment	4499

A.17.10.2.1.2	Test Requirements	4503

A.17.10.2.2	PRS-RSRP reporting delay test case in RRC_IDLE state in FR2 when eDRX cycle > 10.24s	4503

A.17.10.2.2.1	Test Purpose and Environment	4503

A.17.10.2.2.2	Test Requirements	4503

A.17.11	 Measurement Performance Requirements for RedCap in RRC_IDLE	4504

A.17.11.1	RSTD Measurements	4504

A.17.11.1.1	RSTD measurement accuracy test case for RedCap UE without FH in FR2 in RRC_IDLE state without eDRX	4504

A.17.11.1.1.1	Test purpose and Environment	4504

11.1.1.2	Test Requirements	4506

A.17.11.1.2	RSTD without FH measurement accuracy test case for single positioning frequency layer in FR2 SA in RRC_IDLE state with eDRX > 10.24s	4506

A.17.11.1.2.1	Test purpose and environment	4506

A.17.11.1.2.2	Test requirements	4508

A.17.11.2	PRS-RSRP Measurements	4508

A.17.11.2.1	PRS-RSRP measurement accuracy test case for RedCap UE in FR2 in RRC_IDLE state	4508

A.17.11.2.1.1	Test Purpose and Environment	4508

A.17.11.2.1.2	Test parameters	4508

A.17.11.2.2	PRS-RSRP measurement accuracy test case in RRC_IDLE state in FR2 when eDRX cycle > 10.24s	4510

A.17.11.2.2.1	Test purpose and Environment	4510

A.17.11.2.2.1	Test parameters	4511

A.17.11.2.2.2	Test Requirements	4511

A.18	E-UTRA standalone tests for NR RRM for RedCap	4511

A.18.1	RRC_IDLE state mobility	4511

A.18.1.1	Inter-RAT NR Cell re-selection	4511

A.18.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	4511

A.18.1.1.1.1	Test Purpose and Environment	4511

A.18.1.1.1.2	Test Requirements	4514

A.18.2	RRC_CONNECTED state mobility	4514

A.18.2.1	Handover	4514

A.18.2.1.1	E-UTRAN - NR handover in FR1	4514

A.18.2.1.1.1	Test Purpose and Environment	4514

A.18.2.1.1.2	Test Requirements	4518

A.18.2.2	RRC connection release with redirection	4518

A.18.2.2.1	Redirection from E-UTRA to NR FR1 for redcap UE	4518

A.18.2.2.1.1	Test Purpose and Environment	4518

A.18.2.2.1.2	Test Parameters	4518

A.18.2.2.1.3	Test Requirements	4521

A.18.3	Measurement procedure	4522

A.18.3.1	E-UTRA – NR Inter-RAT Measurements	4522

A.18.3.1.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4522

A.18.3.1.1.1	Test Purpose and Environment	4522

A.18.3.1.1.2	Test Requirements	4525

A.18.3.1.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4525

A.18.3.1.2.1	Test Purpose and Environment	4525

A.18.3.1.2.2	Test Requirements	4529

A.18.3.1.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4529

A.18.3.1.3.1	Test Purpose and Environment	4529

A.18.3.1.3.2	Test Requirements	4533

A.18.3.1.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4533

A.18.3.1.4.1	Test Purpose and Environment	4533

A.18.3.1.4.2	Test Requirements	4537

A.18.3.1.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	4537

A.18.3.1.5.1	Test Purpose and Environment	4537

A.18.3.1.5.2	Test Requirements	4539

A.18.3.1.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	4539

A.18.3.1.6.1	Test Purpose and Environment	4539

A.18.3.1.6.2	Test Requirements	4541

A.18.3.1.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	4542

A.18.3.1.7.1	Test Purpose and Environment	4542

A.18.3.1.7.2	Test Requirements	4543

A.18.3.1.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	4544

A.18.3.1.8.1	Test Purpose and Environment	4544

A.18.3.1.8.2	Test Requirements	4546

A.19	NR standalone tests for ATG	4547

A.19.1	RRC_IDLE state mobility	4547

A.19.1.1	Cell reselection to FR1 intra-frequency NR case	4547

A.19.1.1.1	Test Purpose and Environment	4547

A.19.1.1.2	Test Parameters	4547

A.19.1.1.3	Test Requirements	4548

A.19.1.2	Cell reselection to FR1 inter-frequency NR case	4548

A.19.1.2.1	Test Purpose and Environment	4548

A.19.1.2.2	Test Parameters	4548

A.19.1.2.3	Test Requirements	4550

A.19.1.3	Cell reselection to FR1 inter-frequency NR case for UE configured with hs-ATG-cellReselectionSet-r18	4551

A.19.1.3.1	Test Purpose and Environment	4551

A.19.1.3.2	Test Parameters	4551

A.19.1.3.3	Test Requirements	4553

A.19.2	RRC_CONNECTED state mobility	4554

A.19.2.1	Handover	4554

A.19.2.1.1	Intra-frequency handover from FR1 to FR1; known target cell	4554

A19.2.1.1.1	Test Purpose and Environment	4554

A.19.2.1.1.2	Test Parameters	4554

A.19.2.1.2.3	Test Requirements	4554

A.19.2.1.2	Inter-frequency handover from FR1 to FR1; unknown target cell	4555

A.19.2.1.2.1	Test Purpose and Environment	4555

A.19.2.1.2.2	Test Parameters	4555

A.19.2.1.2.3	Test Requirements	4556

A.19.2.2	Conditional Handover	4556

A.19.2.2.1	Intra-frequency distance-based conditional Handover from FR1 to FR1	4556

A.19.2.2.1.1	Test Purpose and Environment	4556

A.19.2.2.1.2	Test Parameters	4556

A.19.2.2.1.3	Test Requirements	4558

A.19.2.2.2	Inter-frequency distance-based conditional Handover from FR1 to FR1	4559

A.19.2.2.2.1	Test Purpose and Environment	4559

A.19.2.2.2.2	Test Parameters	4559

A.19.2.2.2.3	Test Requirements	4561

A.19.2.3	RRC Connection Mobility Control	4561

A.19.2.3.1	SA: RRC Re-establishment	4561

A.19.2.3.1.1	Intra-frequency RRC Re-establishment in FR1 for ATG	4561

A.19.2.3.1.1.1	Test Purpose and Environment	4562

A.19.2.3.1.1.2 Test Requirements	4562

A.19.2.3.1.2	Inter-frequency RRC Re-establishment in FR1 with unknown target cell without serving cell timing for ATG	4562

A.19.2.3.1.2.1	Test Purpose and Environment	4562

A.19.2.3.1.2.2	Test Requirements	4564

A.19.2.3.2	Random Access for ATG UE	4565

A.19.2.3.2.1.1	Test Purpose and Environment	4565

A.19.2.3.2.1.2	Test Requirements	4566

A.19.2.3.2.2.1	Test Purpose and Environment	4566

A.19.2.3.2.2.2	Test Requirements	4566

A.19.2.3.2.3	2-step RA type contention based random access test in FR1 for NR standalone	4566

A.19.2.3.2.3.1	Test Purpose and Environment	4566

A.19.2.3.2.3.2	Test Requirements	4567

A.19.2.3.2.4	2-step RA type non-contention based test in FR1 for NR standalone	4567

A.19.2.3.2.4.1	Test Purpose and Environment	4567

A.19.2.3.2.4.2	Test Requirements	4568

A.19.2.3.3.1.1	Test Purpose and Environment	4568

A.19.2.3.3.1.2	Test Parameters	4568

A.19.2.3.3.1.3	Test Requirements	4568

A.19.3	Timing	4569

A.19.3.1	UE transmit timing	4569

A.19.3.1.1	ATG UE Transmit Timing Test for FR1	4569

A.19.3.1.1.1	Test Purpose and environment	4569

A.19.3.1.1.2	Test requirements	4570

A.19.3.2	UE timer accuracy	4571

A.19.3.3	Timing advance	4571

A.19.3.3.1	SA FR1 timing advance adjustment accuracy	4571

A.19.3.3.1.1	Test Purpose and Environment	4571

A.19.3.3.1.2	Test Parameters	4571

A.19.3.3.1.3	Test Requirements	4572

A.19.4	Signalling characteristics	4572

A.19.4.1	Radio link Monitoring	4572

A.19.4.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	4572

A.19.4.1.1.1	Test Purpose and Environment	4572

A.19.4.1.1.2	Test Requirements	4575

A.19.4.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	4575

A.19.4.1.2.1	Test Purpose and Environment	4575

A.19.4.1.2.2	Test Requirements	4578

A.19.4.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	4578

A.19.4.1.3.1	Test Purpose and Environment	4578

A.19.4.1.3.2	Test Requirements	4581

A.19.4.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	4582

A.19.4.1.4.1	Test Purpose and Environment	4582

A.19.4.1.4.2	Test Requirements	4585

A.19.4.2	Beam Failure Detection and Link recovery procedures	4585

A.19.4.2.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	4585

A.19.4.2.1.1	Test Purpose and Environment	4585

A.19.4.2.1.2	Test Requirements	4589

A.19.4.2.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	4589

A.19.4.2.2.1	Test Purpose and Environment	4589

A.19.4.2.2.2	Test Requirements	4593

A.19.4.3	Active BWP switch	4593

A.19.4.3.1	DCI-based and Timer-based Active BWP Switch	4593

A.19.4.3.1.1	NR FR1 DL active BWP switch with non-DRX in SA	4593

A.19.4.3.2	RRC-based Active BWP Switch	4596

A.19.4.3.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	4596

A.19.4.4	UE specific CBW change	4598

A19.4.4.1	UE specific CBW change on PCell in FR1 in non-DRX	4598

A19.4.4.1.1	Test Purpose and Environment	4598

A.19.4.4.1.2	Test Requirements	4601

A.19.4.5	Pathloss reference signal switching delay	4601

A.19.4.5.1	MAC-CE based pathloss reference signal switch delay	4601

A.19.4.5.1.1	Test Purpose and Environment	4601

A.19.4.5.1.2	Test Requirements	4603

A.19.5	Measurement procedure	4604

A.19.5.1	Intra-frequency Measurements	4604

A.19.5.1.1	SA event triggered reporting tests without gap without SSB index reading under non-DRX	4604

A.19.5.1.1.1	Test purpose and Environment	4604

A.19.5.1.1.2	Test parameters	4604

A.19.5.1.1.3	Test Requirements	4604

A.19.5.1.2	SA event triggered reporting tests with per-UE gaps under non-DRX	4605

A.19.5.1.2.1	Test purpose and Environment	4605

A.19.5.1.2.2	Test parameters	4605

A.19.5.1.2.3	Test Requirements	4605

A.19.5.1.3	SA event triggered reporting tests without gap under non-DRX with SSB index reading	4605

A.19.5.1.3.1	Test purpose and Environment	4605

A.19.5.1.3.2	Test parameters	4606

A.19.5.1.3.3	Test Requirements	4606

A.19.5.1.4	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading	4606

A.19.5.1.4.1	Test purpose and Environment	4606

A.19.5.1.4.2	Test parameters	4606

A.19.5.1.4.3	Test Requirements	4607

A.19.5.2	Inter-frequency Measurements	4607

A.19.5.2.1.2	Test parameters	4607

A.19.5.2.1.3	Test Requirements	4608

A.19.5.2.2.2	Test parameters	4609

A.19.5.2.3.2	Test parameters	4609

A.19.5.2.3.3	Test Requirements	4610

A.19.5.3	L1-RSRP measurement for beam reporting for ATG	4610

A.19.5.3.1	SSB based L1-RSRP measurement when DRX is not used	4610

A.19.5.3.1.1	Test Purpose and Environment	4610

A.19.5.3.1.2	Test parameters	4610

A.19.5.3.1.3	Test Requirements	4611

A.19.5.3.2	CSI-RS based L1-RSRP measurement when DRX is not used	4611

A.19.5.3.2.1	Test Purpose and Environment	4611

A.19.5.3.2.2	Test parameters	4611

A.19.5.3.2.3	Test Requirements	4611

A.19.5.4	L1-SINR measurement for beam reporting for ATG	4611

A.19.5.4.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is not used	4611

A.19.5.4.1.3	Test Requirements	4612

A.19.5.4.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	4612

A.19.5.4.2.1	Test Purpose and Environment	4612

A.19.5.4.2.2	Test parameters	4612

A.19.5.4.2.3	Test Requirements	4613

A.19.5.4.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	4613

A.19.5.4.3.1	Test Purpose and Environment	4613

A.19.5.4.3.2	Test parameters	4613

A.19.5.4.3.3	Test Requirements	4613

A.19.5.5	NR measurements with autonomous gaps for ATG	4614

A.19.5.5.1	SA intra-frequency CGI identification of NR neighbor cell in FR1	4614

A.19.5.5.1.1	Test Purpose and Environment	4614

A.19.5.5.1.2	Test Parameters	4614

A.19.5.5.1.3	Test Requirements	4614

A.19.6	Measurement Performance requirements	4614

A.19.6.1	SS-RSRP for ATG UE	4615

A.19.6.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	4615

A.19.6.1.1.1	Test Purpose and Environment	4615

A.19.6.1.1.2	Test parameters	4615

A.19.6.1.1.3	Test Requirements	4615

A.19.6.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	4615

A.19.6.1.2.1	Test Purpose and Environment	4615

A.19.6.1.2.2	Test parameters	4616

A.19.6.1.2.3	Test Requirements	4616

A.19.6.2	SS-RSRQ for ATG UE	4616

A.19.6.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	4616

A.19.6.2.1.1	Test Purpose and Environment	4616

A.19.6.2.1.2	Test Parameters	4616

A.19.6.2.1.3	Test Requirements	4617

A.19.6.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	4617

A.19.6.2.2.1	Test Purpose and Environment	4617

A.19.6.2.2.2	Test Parameters	4617

A.19.6.2.2.3	Test Requirements	4617

A.19.6.3	SS-SINR for ATG UE	4618

A.19.6.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	4618

A.19.6.3.1.1	Test Purpose and Environment	4618

A.19.6.3.1.2	Test Parameters	4618

A.19.6.3.1.3	Test Requirements	4618

A.19.6.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	4618

A.19.6.3.2.1	Test Purpose and Environment	4618

A.19.6.3.2.2	Test Parameters	4618

A.19.6.3.2.3	Test Requirements	4619

A.19.6.4	L1-RSRP measurement for beam reporting for ATG UE	4619

A.19.6.4.1	SSB based L1-RSRP measurement	4619

A.19.6.4.1.1	Test Purpose and Environment	4619

A.19.6.4.1.2	Test parameters	4619

A.19.6.4.1.3	Test Requirements	4620

A.19.6.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	4620

A.19.6.4.2.1	Test Purpose and Environment	4620

A.19.6.4.2.2	Test parameters	4620

A.19.6.4.2.3	Test Requirements	4620

A.19.6.5	L1-SINR measurement for beam reporting based CMR for ATG UE	4621

A.19.6.5.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	4621

A.19.6.5.1.1	Test Purpose and Environment	4621

A.19.6.5.1.2	Test parameters	4621

A.19.6.5.1.3	Test Requirements	4621

A.19.6.5.2	L1-SINR measurement with SSB based CMR and dedicated IMR	4621

A.19.6.5.2.1	Test Purpose and Environment	4621

A.19.6.5.2.2	Test parameters	4622

A.19.6.5.2.3	Test Requirements	4622

A.19.6.5.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	4622

A.19.6.5.3.1	Test Purpose and Environment	4622

A.19.6.5.3.2	Test parameters	4622

A.19.6.5.3.3	Test Requirements	4623

A.19.6.6	CSI-RSRP for ATG UE	4623

A.19.6.6.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	4623

A.19.6.6.1.1	Test Purpose and Environment	4623

A.19.6.6.1.2	Test parameters	4623

A.19.6.6.1.3	Test Requirements	4623

A.19.6.6.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	4624

A.19.6.6.2.1	Test Purpose and Environment	4624

A.19.6.6.2.2	Test parameters	4624

A.19.6.6.2.3	Test Requirements	4624

A.19.6.7	CSI-RSRQ for ATG UE	4624

A.19.6.7.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	4624

A.19.6.7.1.1	Test Purpose and Environment	4624

A.19.6.7.1.2	Test Parameters	4624

A.19.6.7.1.3	Test Requirements	4625

A.19.6.7.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	4625

A.19.6.7.2.1	Test Purpose and Environment	4625

A.19.6.7.2.2	Test Parameters	4625

A.19.6.7.2.3	Test Requirements	4626

A.19.6.8	CSI-SINR for ATG UE	4626

A.19.6.8.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	4626

A.19.6.8.1.1	Test Purpose and Environment	4626

A.19.6.8.1.2	Test Parameters	4626

A.19.6.8.1.3	Test Requirements	4626

A.19.6.8.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	4627

A.19.6.8.2.1	Test Purpose and Environment	4627

A.19.6.8.2.2	Test Parameters	4627

A.19.6.8.2.3	Test Requirements	4627

A.4.7	Measurement Performance requirements	4628

A.4.7.1	SS-RSRP	4628

A.4.7.1.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	4628

A.4.7.1.1.1	Test Purpose and Environment	4628

A.4.7.1.1.2	Test parameters	4628

A.4.7.1.1.3	Test Requirements	4633

A.4.7.1.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	4633

A.4.7.1.2.1	Test Purpose and Environment	4633

A.4.7.1.2.2	Test parameters	4633

A.4.7.1.2.3	Test Requirements	4636

A.4.7.1.3	Void	4636

A.4.7.2	SS-RSRQ	4636

A.4.7.2.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	4636

A.4.7.2.1.1	Test Purpose and Environment	4636

A.4.7.2.1.2	Test Parameters	4636

A.4.7.2.1.3	Test Requirements	4640

A.4.7.2.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	4640

A.4.7.2.2.1	Test Purpose and Environment	4640

A.4.7.2.2.2	Test Parameters	4640

A.4.7.2.2.3	Test Requirements	4644

A.4.7.3	SS-SINR	4644

A.4.7.3.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	4644

A.4.7.3.1.1	Test Purpose and Environment	4644

A.4.7.3.1.2	Test Parameters	4644

A.4.7.3.1.3	Test Requirements	4648

A.4.7.3.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	4648

A.4.7.3.2.1	Test Purpose and Environment	4648

A.4.7.3.2.2	Test Parameters	4648

A.4.7.3.2.3	Test Requirements	4651

A.4.7.4	L1-RSRP measurement for beam reporting	4651

A.4.7.4.1	SSB based L1-RSRP measurement	4651

A.4.7.4.1.1	Test Purpose and Environment	4651

A.4.7.4.1.2	Test parameters	4651

A.4.7.4.1.3	Test Requirements	4654

A.4.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	4654

A.4.7.4.2.1	Test Purpose and Environment	4654

A.4.7.4.2.2	Test parameters	4655

A.4.7.4.2.3	Test Requirements	4658

A.4.7.5	SFTD accuracy	4658

A.4.7.5.1	SFTD accuracy	4658

A.4.7.5.1.1	Test Purpose and Environment	4658

A.4.7.5.1.2	Test Parameters	4658

A.4.7.5.1.3	Test Requirements	4661

A.4.7.5.2	Void	4661

A.4.7.5.3	Void	4661

A.4.7.6	CLI measurements	4661

A.4.7.6.1	EN-DC SRS-RSRP measurement accuracy with FR1 serving cell	4661

A.4.7.6.1.1	Test Purpose and Environment	4661

A.4.7.6.1.2	Test parameters	4661

A.4.7.6.1.3	Test Requirements	4664

A.4.7.6.2	EN-DC CLI-RSSI measurement accuracy with FR1 serving cell	4664

A.4.7.6.2.1	Test Purpose and Environment	4664

A.4.7.6.2.2	Test parameters	4665

A.4.7.6.2.3	Test Requirements	4666

A.4.7.7	L1-SINR measurement for beam reporting	4666

A.4.7.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR	4670

A.4.7.7.2.1	Test Purpose and Environment	4670

A.4.7.7.2.2	Test parameters	4670

A.4.7.7.2.3	Test Requirements	4673

A.4.7.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	4673

A.4.7.7.3.1	Test Purpose and Environment	4673

A.4.7.7.3.2	Test parameters	4674

A.4.7.7.3.3	Test Requirements	4677

A.4.7.8	CSI-RSRP	4677

A.4.7.8.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	4677

A.4.7.8.1.1	Test Purpose and Environment	4677

A.4.7.8.1.2	Test parameters	4677

A.4.7.8.1.3	Test Requirements	4681

A.4.7.8.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	4681

A.4.7.8.2.1	Test Purpose and Environment	4681

A.4.7.8.2.2	Test parameters	4682

A.4.7.8.2.3	Test Requirements	4685

A.4.7.9	CSI-RSRQ	4685

A.4.7.9.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	4685

A.4.7.9.1.1	Test Purpose and Environment	4685

A.4.7.9.1.2	Test Parameters	4685

A.4.7.9.1.3	Test Requirements	4689

A.4.7.9.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	4689

A.4.7.9.2.1	Test Purpose and Environment	4689

A.4.7.9.2.2	Test Parameters	4689

A.4.7.9.2.3	Test Requirements	4693

A.4.7.10	CSI-SINR	4693

A.4.7.10.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	4693

A.4.7.10.1.1	Test Purpose and Environment	4693

A.4.7.10.1.2	Test Parameters	4693

A.4.7.10.1.3	Test Requirements	4696

A.4.7.10.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	4697

A.4.7.10.2.1	Test Purpose and Environment	4697

A.4.7.10.2.2	Test Parameters	4697

A.4.7.10.2.3	Test Requirements	4700

A.4.7.11	TDCP amplitude measurement accuracy	4700

A.4.7.11.1	TDCP amplitude measurement accuracy in EN-DC	4700

A.4.7.11.1.1	Test Purpose and Environment	4700

A.4.7.11.1.2	Test parameters	4701

A.4.7.11.1.3	Test Requirements	4702

A.4.8	Void	4702

A.4A	NE-DC test with all NR cells in FR1	4702

A.4A.1	Signaling characteristics	4702

A.4A.1.1	E-UTRAN PSCell addition	4702

A.4A.1.1.1	Test purpose and environment	4702

A.4A.1.1.2	Test Requirements	4706

A.4A.1.2	Active BWP switch	4706

A.4A.1.2.1	E-UTRAN PSCell – NR PCell FR1 DCI-based and Timer-based DL active BWP switch in non-DRX in synchronous NE-DC	4706

A.4A.1.2.1.1	Test Purpose and Environment	4706

A.4A.1.2.1.2	Test Requirements	4709

A.4A.1.3	Intra-frequency handover with E-UTRAN PSCell	4709

A.4A.1.3.1	Test purpose and environment	4709

A.4A.1.3.2	Test Requirements	4713

A.4A.1.4	Handover with PSCell from NE-DC to NE-DC with unknown target PSCell	4713

A.4A.1.4.1	Test Purpose and Environment	4713

A.4A.1.4.2	Test Parameters	4714

A.4A.1.4.3	Test Requirements	4717

A.4A.1.4.3.1	Test Requirements for NR HO	4717

A.4A.1.4.3.2	Test Requirements for LTE PSCell Change	4718

A.4A.2	Measurement performance	4718

A.4A.2.1	SFTD accuracy	4718

A.4A.2.1.1	SFTD accuracy	4718

A.4A.2.1.1.1	Test Purpose	4718

A.4A.2.1.1.2	Test Environment	4718

A.4A.2.1.1.3	Test Requirements	4721

A.5	EN-DC tests with one or more NR cells in FR2	4721

A.5.1	Void	4721

A.5.2	Void	4722

A.5.3	RRC_CONNECTED state mobility	4722

A.5.3.1	Void	4722

A.5.3.2	RRC Connection Mobility Control	4722

A.5.3.2.1	Void	4722

A.5.3.2.2	Random Access	4722

A.5.3.2.2.1	4-step RA type c ontention based random access test in FR2 for PSCell/SCell in EN-DC	4722

A.5.3.2.2.2	4-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	4725

A.5.3.2.2.3	2-step RA type contention based random access test in FR2 for PSCell/SCell in EN-DC	4728

A.5.3.2.2.4	2-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	4731

A.5.3.2.3	Void	4733

A.5.3.3	Handover with PSCell with known FR2 target PSCell	4733

A.5.3.3.1	Test purpose and environment	4733

A.5.3.3.2	Test Requirements	4736

A.5.3.3.3	Void	4736

A.5.3.3.4	Void	4736

A.5.3.3.5	Void	4736

A.5.3.3.6	Void	4737

A.5.4	Timing	4737

A.5.4.1	UE transmit timing	4737

A.5.4.1.1	NR UE Transmit Timing Test for FR2	4737

A.5.4.1.1.1	Test Purpose and environment	4737

A.5.4.1.1.2	Test requirements	4739

A.5.4.1.2	NR UE Transmit Timing Test with 2-TA for FR2 UE supporting multiDCI-IntraCellMultiTRP-TwoTA-r18	4739

A.5.4.1.2.1	Test Purpose and environment	4739

A.5.4.1.2.2	Test requirements	4744

A.5.4.2	UE timer accuracy	4744

A.5.4.3	Timing advance	4744

A.5.4.3.1 EN-DC FR2 timing advance adjustment accuracy	4744

A.5.4.3.1.1 Test Purpose and Environment	4744

A.5.4.3.1.2 Test Parameters	4744

A.5.4.3.1.3	Test Requirements	4747

A.5.5	Signaling characteristics	4747

A.5.5.1	Radio link Monitoring	4747

A.5.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	4747

A.5.5.1.1.1	Test Purpose and Environment	4747

A.5.5.1.1.2	Test Requirements	4750

A.5.5.1.2	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	4750

A.5.5.1.2.1	Test Purpose and Environment	4750

A.5.5.1.2.2	Test Requirements	4753

A.5.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	4754

A.5.5.1.3.1	Test Purpose and Environment	4754

A.5.5.1.3.2	Test Requirements	4756

A.5.5.1.4	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	4756

A.5.5.1.4.1	Test Purpose and Environment	4756

A.5.5.1.4.2	Test Requirements	4759

A.5.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	4759

A.5.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	4762

A.5.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	4765

A.5.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	4768

A.5.5.1.8.2	Test Requirements	4772

A.5.5.1.9	EN-DC Radio Link Monitoring UE Scheduling Restrictions on FR2	4772

A.5.5.1.9.1	Test Purpose and Environment	4772

A.5.5.1.9.2	Test Requirements	4774

A.5.5.1.10	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	4774

A.5.5.1.10.1	Test Purpose and Environment	4774

A.5.5.1.10.2	Test Requirements	4776

A.5.5.1.11	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode for UE supporting fast beam sweeping in multi-Rx	4777

A.5.5.1.11.1	Test Purpose and Environment	4777

A.5.5.1.11.2	Test Requirements	4779

A.5.5.1.12	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode when CD-SSB is outside active BWP	4780

A.5.5.1.12.1	Test Purpose and Environment	4780

A.5.5.1.13	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode when CD-SSB is outside active BWP	4780

A.5.5.1.13.1	Test Purpose and Environment	4780

A.5.5.1.13.2	Test Requirements	4780

A.5.5.1.14	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode for UE supporting NCD-SSB based measurement outside active BWP	4780

A.5.5.1.14.1	Test Purpose and Environment	4780

A.5.5.1.14.2	Test Requirements	4783

A.5.5.2	Interruption	4784

A.5.5.2.1	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	4784

A.5.5.2.1.1	Test Purpose and Environment	4784

A.5.5.2.1.2	Test Requirements	4786

A.5.5.2.2	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	4786

A.5.5.2.2.1	Test Purpose and Environment	4786

A.5.5.2.2.2	Test Requirements	4788

A.5.5.2.3	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	4788

A.5.5.2.3.1	Test Purpose and Environment	4788

A.5.5.2.3.2	Test Requirements	4790

A.5.5.2.4	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	4791

A.5.5.2.4.1	Test Purpose and Environment	4791

A.5.5.2.4.2	Test Requirements	4793

A.5.5.2.5	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	4793

A.5.5.2.5.1	Test Purpose and Environment	4793

A.5.5.2.5.2	Test Requirements	4795

A.5.5.2.6	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	4796

A.5.5.2.6.1	Test Purpose and Environment	4796

A.5.5.2.6.2	Test Requirements	4797

A.5.5.2.7	E-UTRAN – NR FR2 interruptions at E-UTRA SRS carrier based switching	4798

A.5.5.2.7.1	Test Purpose and Environment	4798

A.5.5.2.7.2	Test Requirements	4800

A.5.5.2.8 E-UTRAN – NR FR2 interruptions at NR SRS carrier based switching	4800

A.5.5.2.8.1 Test Purpose and Environment	4800

A.5.5.2.8.3	Test Requirements	4802

A.5.5.2.9	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR PSCell	4802

A.5.5.2.9.1	Test Purpose and Environment	4802

A.5.5.2.9.2	Test Requirements	4805

A.5.5.3	SCell Activation and Deactivation Delay	4805

A.5.5.3.1	SCell Activation and deactivation of SCell in FR2 intra-band	4805

A.5.5.3.1.1	Test Purpose and Environment	4805

A.5.5.3.1.2	Test Requirements	4806

A.5.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 160 ms SCell measurement cycle	4807

A.5.5.3.2.1	Test Purpose and Environment	4807

A.5.5.3.2.2	Test Requirements	4809

A.5.5.3.3	Void	4809

A.5.5.3.4	Void	4809

A.5.5.3.5	SCell Activation and deactivation of SCell in FR2	4809

A.5.5.3.5.1	Test Purpose and Environment	4809

A.5.5.3.5.2	Test Requirements	4812

A.5.5.3.6	Multiple SCell Activation and deactivation of one unknown SCell and one known SCell in FR2	4812

A.5.5.3.6.1	Test Purpose and Environment	4812

A.5.5.3.6.2	Test Requirements	4815

A.5.5.3.7	Direct SCell activation at SCell addition of known SCell in FR2	4815

A.5.5.3.7.1	Test Purpose and Environment	4815

A.5.5.3.7.2	Test Requirements	4818

A.5.5.3.8	Fast SCell Activation of SCell in FR2 intra-band	4818

A.5.5.3.8.1	Test Purpose and Environment	4818

A.5.5.3.8.2	Test Requirements	4821

A.5.5.3.9	PUCCH SCell Activation and deactivation of known SCell in FR2	4821

A.5.5.3.9.1	Test Purpose and Environment	4821

A.5.5.3.9.2	Test Requirements	4824

A.5.5.3.10	PUCCH SCell Activation and deactivation of unknown SCell in FR2	4824

A.5.5.3.10.1	Test Purpose and Environment	4824

A.5.5.3.10.2	Test Requirements	4827

A.5.5.3.11	Multiple SCell activation and deactivation of one known PUCCH SCell and one unknown SCell in FR2	4827

A.5.5.3.11.1	Test Purpose and Environment	4827

A.5.5.3.11.2	Test Requirements	4830

A.5.5.3.12	SCell Activation and deactivation of unknown PUCCH SCell and unknown DL SCell in FR2 in non-DRX	4831

A.5.5.3.12.1	Test Purpose and Environment	4831

A.5.5.3.12.2	Test Requirements	4833

A.5.5.3.13	SCell Activation and deactivation of unknown SCell in FR2 for UE in DRX, capable of small beam sweeping factors and/or short measurement interval	4834

A.5.5.3.13.1	Test Purpose and Environment	4834

A.5.5.3.13.2	Test Requirements	4837

A.5.5.3.14	PUCCH SCell activation and deactivation with FR1 PSCell based on L3 reporting after SCell activation command	4839

A.5.5.3.14.1	Test Purpose and Environment	4839

A.5.5.3.14.2	Test Requirements	4843

A.5.5.3.15	SCell Activation of unknown SCell in FR2 in non-DRX for 160 ms SCell measurement cycle with the L3 reporting during activation	4843

A.5.5.3.15.1	Test Purpose and Environment	4843

A.5.5.3.15.2	Test Requirements	4847

A.5.5.4	Void	4848

A.5.5.5	Beam Failure Detection and Link recovery procedures	4848

A.5.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	4848

A.5.5.5.1.1	Test Purpose and Environment	4848

A.5.5.5.1.2	Test Requirements	4851

A.5.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode	4852

A.5.5.5.2.1	Test Purpose and Environment	4852

A.5.5.5.2.2	Test Requirements	4855

A.5.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	4855

A.5.5.5.3.1	Test Purpose and Environment	4855

A.5.5.5.3.2	Test Requirements	4858

A.5.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	4859

A.5.5.5.4.1	Test Purpose and Environment	4859

A.5.5.5.4.2	Test Requirements	4862

A.5.5.5.5	EN-DC scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	4862

A.5.5.5.5.1	Test Purpose and Environment	4862

A.5.5.5.5.2	Test Requirements	4865

A.5.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	4865

A.5.5.5.6.1	Test Purpose and Environment	4865

A.5.5.5.6.2	Test Requirements	4869

A.5.5.5.7	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	4869

A.5.5.5.7.1	Test Purpose and Environment	4869

A.5.5.5.7.2	Test Requirements	4872

A.5.5.5.8	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	4873

A.5.5.5.8.1	Test Purpose and Environment	4873

A.5.5.5.8.2	Test Requirements	4876

A.5.5.5.9	Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode for UE fulfilling relaxed measurement criterion	4876

A.5.5.5.9.1	Test Purpose and Environment	4876

A.5.5.5.9.2	Test Requirements	4879

A.5.5.6	Active BWP switch	4880

A.5.5.6.1	DCI-based and Timer-based Active BWP Switch	4880

A.5.5.6.1.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	4880

A.5.5.6.1.1.1	Test Purpose and Environment	4880

A.5.5.6.1.1.2	Test Requirements	4882

A.5.5.6.1.2	E-UTRAN – NR PSCell FR2 with FR2 SCell DL active BWP switch in non-DRX in synchronous EN-DC	4883

A.5.5.6.2	RRC-based Active BWP Switch	4886

A.5.5.6.2.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	4886

A.5.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	4889

A.5.5.6.3.1	E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs in synchronous EN-DC	4889

A.5.5.6.4	SCell dormancy switch	4892

A.5.5.6.4.1	E-UTRAN – NR FR2 PSCell SCell dormancy switch of single FR2 SCell inside active time	4892

A.5.5.6.4.1.1	Test Purpose and Environment	4892

A.5.5.6.4.1.2	Test Requirements	4895

A.5.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR2 SCells outside active time	4896

A.5.5.6.4.2.1	Test Purpose and Environment	4896

A.5.5.6.4.2.2	Test Requirements	4900

A.5.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	4900

A.5.5.6.5.1	E-UTRAN – NR PSCell FR2  and NR SCell FR2 DL active BWP switch on multiple CCs with non-DRX in synchronous EN-DC	4900

A.5.5.7	PSCell addition and release delay	4903

A.5.5.7.1	Addition and Release Delay of NR PSCell	4903

A.5.5.7.1.1	Test purpose and environment	4903

A.5.5.7.1.2	Test Requirements	4905

A.5.5.8	Active TCI state switch delay	4905

A.5.5.8.1	MAC-CE based active TCI state switch	4906

A.5.5.8.1.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	4906

A.5.5.8.1.1.1	Test Purpose and Environment	4906

A.5.5.8.1.1.2	Test Requirements	4908

A.5.5.8.2	RRC based active TCI state switch	4909

A.5.5.8.2.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	4909

A.5.5.8.2.1.1	Test Purpose and Environment	4909

A.5.5.8.2.1.2	Test Requirements	4912

A.5.5.9	Uplink spatial relation switch delay	4912

A.5.5.9.1	MAC-CE based uplink spatial relation switch	4912

A.5.5.9.1.1	E-UTRAN – NR PSCell FR2 uplink spatial relation switch for a known spatial relation	4912

A.5.5.9.1.1.1	Test Purpose and Environment	4912

A.5.5.9.1.1.2	Test Requirements	4914

A.5.5.9.2	RRC based spatial relation switch	4914

A.5.5.9.2.1	E-UTRAN – NR PSCell FR2 spatial relation switch associated with a known DL-RS	4914

A.5.5.9.2.1.1	Test Purpose and Environment	4915

A.5.5.9.2.1.2	Test Requirements	4916

A.5.5.10	UE specific CBW change	4917

A.5.5.10.1	UE specific CBW change on FR2 NR PSCell	4917

A.5.5.10.1.1	Test Purpose and Environment	4917

A.5.5.10.1.2	Test Requirements	4919

A.5.5.11	Unified TCI state switch delay	4920

A.5.5.11.1	MAC-CE based active joint TCI state switch	4920

A.5.5.11.1.1	E-UTRAN – NR PSCell FR2 active joint TCI state switch for a known TCI state	4920

A.5.5.11.1.1.1	Test Purpose and Environment	4920

A.5.5.11.1.1.2	Test parameters	4920

A.5.5.11.1.1.3	Test Requirements	4922

A.5.5.11.2	MAC-CE based active uplink TCI state switch	4922

A.5.5.11.2.1	E-UTRAN – NR PSCell FR2 active uplink TCI state switch for a known TCI state	4922

A.5.5.11.2.1.1	Test Purpose and Environment	4922

A.5.5.11.2.1.2	Test parameters	4923

A.5.5.11.2.1.3	Test Requirements	4924

A.5.5.11.3	MAC-CE based active downlink TCI state switch	4925

A.5.5.11.3.1	E-UTRAN – NR PSCell FR2 downlink TCI state switch to cell with additional PCI for a known TCI state	4925

A.5.5.11.3.1.1	Test Purpose and Environment	4925

A.5.5.11.3.1.2	Test Parameters	4925

A.5.5.11.3.1.3	Test Requirements	4928

A.5.5.12	PSCell activation and deactivation delay	4928

A.5.5.12.1	PSCell activation and deactivation delay	4928

A.5.5.12.1.1	Test purpose and environment	4928

A.5.5.12.1.2	Test Requirements	4930

A.5.5.13	Conditional PSCell addition and release delay	4931

A.5.5.13.1	Addition and Release Delay of NR PSCell	4931

A.5.5.13.1.1	Test purpose and environment	4931

A.5.5.13.1.2	Test Requirements	4933

A.5.6	Measurement procedure	4933

A.5.6.1	Intra-frequency Measurements	4933

A.5.6.1.1	EN-DC event triggered reporting test without gap under non-DRX	4933

A.5.6.1.1.1	Test purpose and Environment	4933

A.5.6.1.1.2	Test Requirements	4936

A.5.6.1.2	EN-DC event triggered reporting test without gap under DRX	4936

A.5.6.1.2.1	Test purpose and Environment	4936

A.5.6.1.2.2	Test Requirements	4938

A.5.6.1.3	EN-DC event triggered reporting test with per-UE gaps under non-DRX	4939

A.5.6.1.3.1	Test purpose and Environment	4939

A.5.6.1.3.2	Test Requirements	4941

A.5.6.1.4	EN-DC event triggered reporting test with per-UE gaps under DRX	4941

A.5.6.1.4.1	Test purpose and Environment	4941

A.5.6.1.4.2	Test Requirements	4944

A.5.6.1.5	EN-DC event triggered reporting test without gap under non-DRX when CD-SSB is outside active BWP	4944

A.5.6.1.5.1	Test purpose and Environment	4944

A.5.6.1.5.2	Test Requirements	4944

A.5.6.1.6	EN-DC event triggered reporting test without gap under non-DRX	4944

A.5.6.1.6.1	Test purpose and Environment	4944

A.5.6.1.6.2	Test Requirements	4947

A.5.6.2	Inter-frequency Measurements	4947

A.5.6.2.1 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	4947

A.5.6.2.1.1	Test Purpose and Environment	4947

A.5.6.2.1.2	Test Requirements	4950

A.5.6.2.2 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	4950

A.5.6.2.2.1	Test Purpose and Environment	4950

A.5.6.2.2.2	Test Requirements	4952

A.5.6.2.3 	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	4953

A.5.6.2.3.1	Test Purpose and Environment	4953

A.5.6.2.3.2	Test Requirements	4955

A.5.6.2.4	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	4955

A.5.6.2.4.1	Test Purpose and Environment	4955

A.5.6.2.4.2	Test Requirements	4958

A.5.6.2.5	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	4958

A.5.6.2.5.1	Test Purpose and Environment	4958

A.5.6.2.5.2	Test Requirements	4961

A.5.6.2.6	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	4961

A.5.6.2.6.1	Test Purpose and Environment	4961

A.5.6.2.6.2	Test Requirements	4964

A.5.6.2.7	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	4965

A.5.6.2.7.1	Test Purpose and Environment	4965

A.5.6.2.7.2	Test Requirements	4967

A.5.6.2.8	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	4968

A.5.6.2.8.1	Test Purpose and Environment	4968

A.5.6.2.8.2	Test Requirements	4971

A.5.6.3	L1-RSRP measurement for beam reporting	4971

A.5.6.3.1	SSB based L1-RSRP measurement when DRX is not used	4971

A.5.6.3.1.1	Test Purpose and Environment	4971

A.5.6.3.1.2	Test parameters	4972

A.5.6.3.1.3	Test Requirements	4973

A.5.6.3.2	SSB based L1-RSRP measurement when DRX is used	4973

A.5.6.3.2.1	Test Purpose and Environment	4973

A.5.6.3.2.2	Test parameters	4974

A.5.6.3.2.3	Test Requirements	4975

A.5.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	4975

A.5.6.3.3.1	Test Purpose and Environment	4975

A.5.6.3.3.2	Test parameters	4975

A.5.6.3.3.3	Test Requirements	4977

A.5.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	4977

A.5.6.3.4.1	Test Purpose and Environment	4977

A.5.6.3.4.2	Test parameters	4978

A.5.6.3.4.3	Test Requirements	4979

A.5.6.3.5	CSI-RS based L1-RSRP measurement when DRX is not used and when CD-SSB is outside active BWP	4979

A.5.6.3.5.1	Test Purpose and Environment	4979

A.5.6.3.6	SSB based L1-RSRP measurement when DRX is not used when CD-SSB is outside active BWP	4980

A.5.6.3.6.1	Test Purpose and Environment	4980

A.5.6.3.6.2	Test Requirements	4980

A.5.6.3.7	SSB based L1-RSRP measurement for UE supporting NCD-SSB based L1 measurement outside active BWP when DRX is not used	4980

A.5.6.3.7.1	Test Purpose and Environment	4980

A.5.6.3.7.2	Test parameters	4980

A.5.6.3.7.3	Test Requirements	4982

A.5.6.4	CLI measurements	4982

A.5.6.4.1	SRS-RSRP measurement with DRX	4982

A.5.6.4.1.1	Test Purpose and Environment	4982

A.5.6.4.1.2	Test Parameters	4982

A.5.6.4.1.3	Test Requirements	4984

A.5.6.4.2	CLI-RSSI measurement with DRX	4985

A.5.6.4.2.1	Test Purpose and Environment	4985

A.5.6.4.2.2	Test Parameters	4985

A.5.6.4.2.3	Test Requirements	4986

A.5.6.5	Measurements with autonomous gaps	4986

A.5.6.5.1 	EN-DC inter-frequency CGI identification of NR neighbor cell in FR2	4986

A.5.6.5.1.1	Test Purpose and Environment	4986

A.5.6.5.1.2	Test Requirements	4989

A.5.6.6	L1-SINR measurement for beam reporting	4989

A.5.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	4991

A.5.6.6.2.1	Test Purpose and Environment	4991

A.5.6.6.2.2	Test parameters	4991

A.5.6.6.2.3	Test Requirements	4993

A.5.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	4993

A.5.6.6.3.1	Test Purpose and Environment	4993

A.5.6.6.3.2	Test parameters	4994

A.5.6.6.3.3	Test Requirements	4995

A.5.6.7	CSI-RS based Intra-frequency Measurements	4995

A.5.6.7.1	EN-DC event triggered reporting test without gap under non-DRX	4995

A.5.6.7.1.1	Test purpose and Environment	4995

A.5.6.7.1.2	Test Requirements	4997

A.5.6.8	CSI-RS based Inter-frequency Measurements	4997

A.5.6.8.1 	EN-DC event triggered reporting tests for NR FR2 cell when DRX is used	4997

A.5.6.8.1.1	Test Purpose and Environment	4997

A.5.6.8.1.2	Test Requirements	4999

A.5.7	Measurement Performance requirements	5000

A.5.7.1	SS-RSRP	5000

A.5.7.1.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	5000

A.5.7.1.1.1	Test Purpose and Environment	5000

A.5.7.1.1.2	Test parameters	5000

A.5.7.1.1.3	Test Requirements	5002

A.5.7.1.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	5002

A.5.7.1.2.1	Test Purpose and Environment	5002

A.5.7.1.2.2	Test parameters	5003

A.5.7.1.2.3	Test Requirements	5005

A.5.7.1.3	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	5006

A.5.7.1.3.1	Test Purpose and Environment	5006

A.5.7.1.3.2	Test parameters	5006

A.5.7.1.3.3	Test Requirements	5008

A.5.7.2	SS-RSRQ	5008

A.5.7.2.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	5008

A.5.7.2.1.1	Test Purpose and Environment	5008

A.5.7.2.1.2	Test Parameters	5009

A.5.7.2.1.3	Test Requirements	5010

A.5.7.2.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	5010

A.5.7.2.2.1	Test Purpose and Environment	5010

A.5.7.2.2.2	Test Parameters	5010

A.5.7.2.2.3	Test Requirements	5012

A.5.7.3	SS-SINR	5012

A.5.7.3.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	5012

A.5.7.3.1.1	Test Purpose and Environment	5012

A.5.7.3.1.2	Test Parameters	5012

A.5.7.3.1.3	Test Requirements	5014

A.5.7.3.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	5014

A.5.7.3.2.1	Test Purpose and Environment	5014

A.5.7.3.2.2	Test Parameters	5014

A.5.7.3.2.3	Test Requirements	5015

A.5.7.4	L1-RSRP measurement for beam reporting	5015

A.5.7.4.1	SSB based L1-RSRP measurement	5016

A.5.7.4.1.1	Test Purpose and Environment	5016

A.5.7.4.1.2	Test parameters	5016

A.5.7.4.1.3	Test Requirements	5017

A.5.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	5018

A.5.7.4.2.1	Test Purpose and Environment	5018

A.5.7.4.2.2	Test parameters	5018

A.5.7.4.2.3	Test Requirements	5019

A.5.7.5	CLI measurements	5020

A.5.7.5.1	EN-DC SRS-RSRP measurement accuracy with FR2 serving cell	5020

A.5.7.5.1.1	Test Purpose and Environment	5020

A.5.7.5.1.2	Test parameters	5020

A.5.7.5.1.3	Test Requirements	5022

A.5.7.5.2	EN-DC CLI-RSSI measurement accuracy with FR2 serving cell	5022

A.5.7.5.2.1	Test Purpose and Environment	5022

A.5.7.5.2.2	Test parameters	5022

A.5.7.5.2.3	Test Requirements	5024

A.5.7.6	L1-SINR measurement for beam reporting	5024

A.5.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	5026

A.5.7.6.2.1	Test Purpose and Environment	5027

A.5.7.6.2.2	Test parameters	5027

A.5.7.6.2.3	Test Requirements	5028

A.5.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	5029

A.5.7.6.3.1	Test Purpose and Environment	5029

A.5.7.6.3.2	Test parameters	5029

A.5.7.6.3.3	Test Requirements	5031

A.5.7.7	CSI-RSRP	5031

A.5.7.7.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	5031

A.5.7.7.1.1	Test Purpose and Environment	5031

A.5.7.7.1.2	Test parameters	5031

A.5.7.7.1.3	Test Requirements	5033

A.5.7.7.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	5034

A.5.7.7.2.1	Test Purpose and Environment	5034

A.5.7.7.2.2	Test parameters	5034

A.5.7.7.2.3	Test Requirements	5036

A.5.7.8	CSI-RSRQ	5037

A.5.7.8.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	5037

A.5.7.8.1.1	Test Purpose and Environment	5037

A.5.7.8.1.2	Test Parameters	5037

A.5.7.8.1.3	Test Requirements	5039

A.5.7.8.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	5039

A.5.7.8.2.1	Test Purpose and Environment	5039

A.5.7.8.2.2	Test Parameters	5039

A.5.7.8.2.3	Test Requirements	5041

A.5.7.9	CSI-SINR	5041

A.5.7.9.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	5041

A.5.7.9.1.1	Test Purpose and Environment	5041

A.5.7.9.1.2	Test Parameters	5041

A.5.7.9.1.3	Test Requirements	5043

A.5.7.9.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	5043

A.5.7.9.2.1	Test Purpose and Environment	5043

A.5.7.9.2.2	Test Parameters	5043

A.5.7.9.2.3	Test Requirements	5045

A.5.8	Void	5045

A.6.6	Measurement procedure	5048

A.6.6.1	Intra-frequency Measurements	5048

A.6.6.1.1	SA event triggered reporting tests without gap under non-DRX	5048

A.6.6.1.1.1	Test purpose and Environment	5048

A.6.6.1.1.2	Test parameters	5048

A.6.6.1.1.3	Test Requirements	5050

A.6.6.1.2	SA event triggered reporting tests without gap under DRX	5050

A.6.6.1.2.1	Test purpose and Environment	5050

A.6.6.1.2.2	Test parameters	5050

A.6.6.1.2.3	Test Requirements	5052

A.6.6.1.3	SA event triggered reporting tests with per-UE gaps under non-DRX	5052

A.6.6.1.3.1	Test purpose and Environment	5052

A.6.6.1.3.2	Test parameters	5052

A.6.6.1.3.3	Test Requirements	5054

A.6.6.1.4	SA event triggered reporting tests with per-UE gaps under DRX	5054

A.6.6.1.4.1	Test purpose and Environment	5054

A.6.6.1.4.2	Test parameters	5055

A.6.6.1.4.3	Test Requirements	5057

A.6.6.1.5	SA event triggered reporting tests without gap under non-DRX with SSB index reading	5057

A.6.6.1.5.1	Test purpose and Environment	5057

A.6.6.1.5.2	Test parameters	5057

A.6.6.1.5.3	Test Requirements	5058

A.6.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading	5058

A.6.6.1.6.1	Test purpose and Environment	5058

A.6.6.1.6.2	Test parameters	5059

A.6.6.1.6.3	Test Requirements	5060

A.6.6.1.7	SA event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	5060

A.6.6.1.7.1	Test purpose and Environment	5060

A.6.6.1.7.2	Test parameters	5060

A.6.6.1.7.3	Test Requirements	5062

A.6.6.1.8	SA event triggered reporting tests without gap under DRX for UE configured with highSpeedMeasCA-Scell-r17	5063

A.6.6.1.8.1	Test purpose and Environment	5063

A.6.6.1.8.2	Test parameters	5063

A.6.6.1.8.3	Test Requirements	5065

A.6.6.1.9	SA event triggered reporting tests with MUSIM gap configured	5065

A.6.6.1.9.1	Test purpose and Environment	5065

A.6.6.1.9.2	Test parameters	5065

A.6.6.1.9.3	Test requirements	5067

A.6.6.1.10	SA event triggered reporting tests without gap under non-DRX when CD-SSB is outside active BWP	5067

A.6.6.1.10.1	Test purpose and Environment	5067

A.6.6.1.10.2	Test Requirements	5068

A.6.6.1.11	SA event triggered reporting tests without gap under non-DRX with NCD-SSB	5068

A.6.6.1.11.1	Test purpose and Environment	5068

A.6.6.1.11.2	Test parameters	5068

A.6.6.1.11.3	Test Requirements	5069

A.6.6.1.12	SA event triggered reporting tests without gap under non-DRX with SSB index reading and 12 PRB SSB	5070

A.6.6.1.12.1	Test purpose and Environment	5070

A.6.6.1.12.2	Test parameters	5070

A.6.6.1.12.3	Test Requirements	5070

A.6.6.1.13	SA event triggered reporting tests without gap under Cell DTX	5070

A.6.6.1.13.1	Test purpose and Environment	5071

A.6.6.1.13.2	Test parameters	5071

A.6.6.1.13.3	Test Requirements	5072

A.6.6.1.14	Deactivated PSCell measurement test with 12 PRB SSB bandwidth in FR1	5073

A.6.6.1.14.1	Test Purpose and Environment	5073

A.6.6.1.14.2	Test Parameters	5073

A.6.6.1.14.3	Test Requirements	5076

A.6.6.1.15	SA event triggered reporting test without gap under non-DRX with SSB index reading and 12 PRB SSB for a deactivated SCell	5076

A.6.6.1.15.1	Test purpose and Environment	5076

A.6.6.1.15.2	Test parameters	5076

A.6.6.1.15.3	Test requirements	5078

A.6.6.2	Inter-frequency Measurements	5078

A.6.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	5078

A.6.6.2.1.1	Test Purpose and Environment	5078

A.6.6.2.1.2	Test Requirements	5081

A.6.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used	5081

A.6.6.2.2.1	Test Purpose and Environment	5081

A.6.6.2.2.2	Test Requirements	5084

A.6.6.2.3	Void	5084

A.6.6.2.4	Void	5084

A.6.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	5084

A.6.6.2.5.1	Test Purpose and Environment	5084

A.6.6.2.5.2	Test Requirements	5086

A.6.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used	5086

A.6.6.2.6.1	Test Purpose and Environment	5087

A.6.6.2.6.2	Test Requirements	5089

A.6.6.2.7	Void	5089

A.6.6.2.8	Void	5089

A.6.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern	5089

A.6.6.2.9.1	Test Purpose and Environment	5089

A.6.6.2.9.2	Test Requirements	5091

A.6.6.2.10	SA event triggered reporting tests for FR1 when DRX is used	5092

A.6.6.2.10.1	Test Purpose and Environment	5092

A.6.6.2.10.2	Test Requirements	5094

A.6.6.2.12	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for UE configured with highSpeedMeasInterFreq-r17	5096

A.6.6.2.12.1	Test Purpose and Environment	5096

A.6.6.2.12.2	Test Requirements	5099

A.6.6.2.13	SA event triggered reporting tests for FR1 with measurement gap with priority and periodic MUSIM gap configured	5099

A.6.6.2.13.1	Test Purpose and Environment	5099

A.6.6.2.13.2	Test Requirements	5102

A.6.6.2.14	SA event triggered reporting tests for FR1 with measurement gap without priority and periodic MUSIM gap configured	5102

A.6.6.2.14.1	Test Purpose and Environment	5102

A.6.6.2.14.2	Test Requirements	5105

A.6.6.2.15	SA event triggered reporting tests for FR1 with 3 MHz Channel Bandwidth configured without SSB time index detection when DRX is used	5105

A.6.6.2.15.1	Test Purpose and Environment	5105

A.6.6.2.15.2	Test Requirements	5105

A.6.6.3	Inter-RAT Measurements	5106

A.6.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	5106

A.6.6.3.1.1	Test Purpose and Environment	5106

A.6.6.3.1.2	Test Requirements	5109

A.6.6.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	5109

A.6.6.3.2.1	Test Purpose and Environment	5109

A.6.6.3.2.2	Test Requirements	5112

A.6.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for UE configured with highSpeedMeasFlag-r16	5112

A.6.6.3.3.1	Test Purpose and Environment	5112

A.6.6.3.3.2	Test Requirements	5115

A.6.6.4	L1-RSRP measurement for beam reporting	5116

A.6.6.4.1	SSB based L1-RSRP measurement when DRX is not used	5116

A.6.6.4.1.1	Test Purpose and Environment	5116

A.6.6.4.1.2	Test parameters	5116

A.6.6.4.1.3	Test Requirements	5117

A.6.6.4.2	SSB based L1-RSRP measurement when DRX is used	5117

A.6.6.4.2.1	Test Purpose and Environment	5117

A.6.6.4.2.2	Test parameters	5118

A.6.6.4.2.3	Test Requirements	5119

A.6.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	5119

A.6.6.4.3.1	Test Purpose and Environment	5119

A.6.6.4.3.2	Test parameters	5120

A.6.6.4.3.3	Test Requirements	5121

A.6.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	5121

A.6.6.4.4.1	Test Purpose and Environment	5121

A.6.6.4.4.2	Test parameters	5121

A.6.6.4.4.3	Test Requirements	5123

A.6.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	5123

A.6.6.4.5.1	Test Purpose and Environment	5123

A.6.6.4.5.2	Test parameters	5123

A.6.6.4.5.3	Test Requirements	5125

A.6.6.4.6	Inter-cell SSB based L1-RSRP measurements on FR1 PCell when DRX is used	5125

A.6.6.4.6.1	Test Purpose and Environment	5125

A.6.6.4.6.2	Test parameters	5125

A.6.6.4.6.3	Test Requirements	5127

A.6.6.4.7	SSB based L1-RSRP measurement when DRX is not used when CD-SSB is outside active BWP	5127

A.6.6.4.7.1	Test Purpose and Environment	5127

A.6.6.4.7.2	Test Requirements	5128

A.6.6.4.8	CSI-RS based L1-RSRP measurement when DRX is not used when CD-SSB is outside active BWP	5128

A.6.6.4.8.1	Test Purpose and Environment	5128

A.6.6.4.9	SSB based L1-RSRP measurement for UE supporting NCD-SSB based L1 measurement outside active BWP when DRX is not used	5128

A.6.6.4.9.1	Test Purpose and Environment	5128

A.6.6.4.9.2	Test parameters	5128

A.6.6.4.9.3	Test Requirements	5130

A.6.6.5	Inter-RAT UTRAN FDD measurements	5130

A.6.6.5.1	SA NR - UTRAN FDD event-triggered reporting in non-DRX in FR1	5130

A.6.6.5.1.1	Test Purpose and Environment	5130

A.6.6.5.1.2	Test Requirements	5132

A.6.6.6	CLI measurements	5133

A.6.6.6.1	SRS-RSRP measurement with DRX	5133

A.6.6.6.1.1	Test Purpose and Environment	5133

A.6.6.6.1.2	Test Parameters	5133

A.6.6.6.1.3	Test Requirements	5135

A.6.6.6.2	CLI-RSSI measurement with DRX	5135

A.6.6.6.2.1	Test Purpose and Environment	5135

A.6.6.6.2.2	Test Parameters	5136

A.6.6.6.2.3	Test Requirements	5137

A.6.6.7	NR measurements with autonomous gaps	5137

A.6.6.7.1	SA intra-frequency CGI identification of NR neighbor cell in FR1	5137

A.6.6.7.1.1	Test Purpose and Environment	5137

A.6.6.7.1.2	Test Parameters	5137

A.6.6.7.1.3	Test Requirements	5140

A.6.6.7.2	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA	5140

A.6.6.7.2.1	Test Purpose and Environment	5140

A.6.6.7.2.2	Test Requirements	5142

A.6.6.8	L1-SINR measurement for beam reporting	5143

A.6.6.8.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is used	5143

A.6.6.8.1.1	Test Purpose and Environment	5143

A.6.6.8.1.2	Test parameters	5143

A.6.6.8.1.3	Test Requirements	5145

A.6.6.8.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	5145

A.6.6.8.2.1	Test Purpose and Environment	5145

A.6.6.8.2.2	Test parameters	5145

A.6.6.8.2.3	Test Requirements	5147

A.6.6.8.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	5147

A.6.6.8.3.1	Test Purpose and Environment	5147

A.6.6.8.3.2	Test parameters	5148

A.6.6.8.3.3	Test Requirements	5149

A.6.6.9	Idle Mode CA/DC Measurements	5149

A.6.6.9.1	SA Idle mode CA/DC measurement for FR1	5149

A.6.6.9.1.1	Test Purpose and Environment	5149

A.6.6.9.1.2	Test Requirements	5153

A.6.6.9.2	 Idle mode fast CA/DC eEMR measurement for FR1 without valid reporting	5153

A.6.6.9.2.1	Test Purpose and Environment	5153

A.6.6.9.2.2	Test Requirements	5156

A.6.6.9.3	Idle mode fast CA/DC cell reselection measurement for FR1 without valid reporting	5156

A.6.6.9.3.1	Test Purpose and Environment	5156

A.6.6.9.3.2	Test Requirements	5159

A.6.6.9.4	Idle mode fast CA/DC cell reselection measurement for FR1 with valid reporting	5159

A.6.6.9.4.1	Test Purpose and Environment	5159

A.6.6.9.4.2	Test Requirements	5162

A.6.6.9.5	SA Idle mode CA/DC measurement for FR1 with 12RB SSB	5162

A.6.6.9.5.1	Test Purpose and Environment	5162

A.6.6.9.5.2	Test Requirements	5163

A.6.6.10	CSI-RS based intra-frequency Measurements	5163

A.6.6.10.1	SA event triggered reporting tests without gap under non-DRX	5163

A.6.6.10.1.1	Test purpose and Environment	5163

A.6.6.10.1.2	Test Requirements	5165

A.6.6.11	CSI-RS based inter-frequency Measurements	5165

A.6.6.11.1	 SA event triggered reporting tests with gap under DRX	5165

A.6.6.11.1.1	Test Purpose and Environment	5165

A.6.6.11.1.2	Test Requirements	5167

A.6.6.12	RSTD measurements	5168

A.6.6.12.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA	5168

A.6.6.12.1.1	Test Purpose and Environment	5168

A.6.6.12.1.2	Test Requirements	5171

A.6.6.12.2	NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR1 SA	5171

A.6.6.12.2.1	Test Purpose and Environment	5172

A.6.6.12.2.2	Test Requirements	5175

A.6.6.12.3	NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA	5175

A.6.6.12.3.1	Test Purpose and Environment	5175

A.6.6.12.3.2	Test Requirements	5178

A.6.6.12.4	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA without measurement gap	5178

A.6.6.12.4.1	Test Purpose and Environment	5178

A.6.6.12.4.2	Test Requirements	5182

A.6.6.12.5	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_CONNECTED state with Rx TEG	5182

A.6.6.12.5.1	Test Purpose and Environment	5182

A.6.6.12.5.2	Test Requirements	5185

A.6.6.12.6	NR RSTD measurement reporting delay test case for PRS aggregation in FR1 SA in RRC_CONNECTED mode	5185

A.6.6.12.6.1	Test Purpose and Environment	5185

A.6.6.12.6.2	Test Requirements	5192

A.6.6.13 PRS-RSRP measurements	5192

A.6.6.13.1	PRS-RSRP reporting delay test case for single positioning frequency layer	5192

A.6.6.13.1.1	Test purpose and Environment	5192

A.6.6.13.1.2	Test Requirements	5194

A.6.6.13.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	5194

A.6.6.13.2.1	Test purpose and Environment	5194

A.6.6.13.2.2	Test Requirements	5197

A.6.6.13.3	PRS-RSRP reporting delay test case for reduced number of samples	5197

A.6.6.13.3.1	Test purpose and Environment	5197

A.6.6.13.3.2	Test Requirements	5199

A.6.6.13.4	PRS-RSRP reporting delay test case for single positioning frequency layer outside MG	5199

A.6.6.13.4.1	Test purpose and Environment	5199

A.6.6.14	UE Rx-Tx time difference measurements	5202

A.6.6.14.1	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	5202

A.6.6.14.1.1	Test purpose and environment	5202

A.6.6.14.1.2	Test requirements	5204

A.6.6.14.2	UE Rx-Tx time difference measurement for dual positioning frequency layers in FR1 SA	5204

A.6.6.14.2.1	Test purpose and environment	5204

A.6.6.14.2.2	Test requirements	5206

A.6.6.14.3	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA with reduced sample number	5207

A.6.6.14.3.1	Test purpose and environment	5207

A.6.6.14.3.2	Test requirements	5209

A.6.6.14.4	UE Rx-Tx time difference measurement without gaps in FR1 SA	5209

A.6.6.14.4.1	Test purpose and environment	5209

A.6.6.14.4.2	Test requirements	5211

A.6.6.14.5	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA with multiple RxTx TEGs	5211

A.6.6.14.4.1	Test purpose and environment	5211

A.6.6.14.4.2	Test requirements	5213

A.6.6.14.6	UE Rx-Tx time difference measurements with PRS bandwidth aggregation in FR1 SA	5214

A.6.6.14.6.1	Test purpose and environment	5214

A.6.6.14.6.2	Test requirements	5217

A.6.6.15	Idle Mode measurements of inter-RAT DC candidate cells for early reporting	5217

A.6.6.15.1	Test Purpose and Environment	5217

A.6.6.15.2	Test Requirements	5221

A.6.6.16	PRS-RSRPP measurements	5222

A.6.6.16.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_CONNECTED state	5222

A.6.6.16.1.1	Test purpose and Environment	5222

A.6.6.16.1.2	Test Requirements	5224

A.6.6.16.2	PRS-RSRPP reporting delay test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_CONNECTED state	5224

A.6.6.16.2.1	Test purpose and Environment	5224

A.6.6.16.2.2	Test Requirements	5226

A.6.6.16.3	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_CONNECTED state without measurement gap	5226

A.6.6.16.3.1	Test purpose and Environment	5226

A.6.6.16.3.2	Test Requirements	5228

A.6.6.17	SA event triggered reporting tests with Pre-MG	5229

A.6.6.17.1	SA event triggered reporting tests with autonomous activation/deactivation Pre-MG	5229

A.6.6.17.1.1	Test purpose and Environment	5229

A.6.6.17.1.2	Test parameters	5229

A.6.6.17.1.3	Test Requirements	5231

A.6.6.17.2	SA event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation	5232

A.6.6.17.2.1	Test purpose and Environment	5232

A.6.6.17.2.2	Test parameters	5232

A.6.6.17.2.3	Test Requirements	5234

A.6.6.17.3	Void	5235

A.6.6.17.3.1	Void	5235

A.6.6.17.3.2	Void	5235

A.6.6.17.3.3	Void	5235

A.6.6.18	SA event triggered reporting tests with concurrent gaps	5235

A.6.6.18.1	SA event triggered reporting tests for FR1 concurrent gaps with non-overalpping scenario for SSB-based measurements in both inter-frequency layers	5235

A.6.6.18.1.1	Test Purpose and Environment	5235

A.6.6.18.1.2	Test Requirements	5237

A.6.6.18.2	SA event triggered reporting tests for FR1 concurrent gap with partially partial overalpping scenario for SSB-based measurements in both inter-frequency layers	5237

A.6.6.18.2.1	Test Purpose and Environment	5237

A.6.6.18.2.2	Test Requirements	5240

A.6.6.18.3	SA NR - E-UTRAN and NR FR1 concurrent event-triggered reporting in non-DRX in FR1	5240

A.6.6.18.3.1	Test Purpose and Environment	5240

A.6.6.18.3.2	Test Requirements	5244

A.6.6.18.4	SA event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used	5245

A.6.6.18.4.1	Test Purpose and Environment	5245

A.6.6.18.4.2	Test Requirements	5247

A.6.6.19	SA event triggered reporting tests with NCSG	5248

A.6.6.19.1	SA event triggered reporting tests with NCSG under non-DRX in FR1	5248

A.6.6.19.1.1	Test purpose and Environment	5248

A.6.6.19.1.2	Test parameters	5248

A.6.6.19.1.3	Test Requirements	5250

A.6.6.19.2	SA event triggered reporting tests for FR1 with NCSG for inter-frequency measurement	5250

A.6.6.19.2.1	Test Purpose and Environment	5250

A.6.6.19.2.2	Test parameters	5250

A.6.6.19.2.3	Test Requirements	5253

A.6.6.19.3	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 with NCSG	5253

A.6.6.19.3.1	Test Purpose and Environment	5253

A.6.6.19.3.2	Test parameters	5253

A.6.6.19.3.3	Test Requirements	5256

A.6.6.19.4	Event triggered reporting on SCC with deactivated SCell test with per-UE NCSG under non-DRX	5257

A.6.6.19.4.1	Test purpose and Environment	5257

A.6.6.19.4.2	Test parameters	5257

A.6.6.19.4.3	Test Requirements	5259

A.6.6.20	UE Rx-Tx time difference measurement for propagation delay compensation	5259

A.6.6.20.1	Test purpose and environment	5259

A.6.6.20.2	Test requirements	5261

A.6.6.21	UE Rx-Tx time difference measurement with TRS for RTT-based PDC in FR1 SA	5261

A.6.6.21.1	Test purpose and environment	5261

A.6.6.21.2	Test requirements	5263

A.6.6.22	SA event triggered reporting tests for concurrent measurement gaps with Pre-MG	5263

A.6.6.22.1	SA event triggered reporting tests for FR1 concurrent gap with Pre-MG with partially partial overalpping scenario for SSB-based measurements in both intra-frequency and inter-frequency layers	5263

A.6.6.22.1.1	Test Purpose and Environment	5263

A.6.6.22.1.2	Test Requirements	5266

A.6.6.22.2	SA event triggered reporting tests for concurrent gap with pre-configured gaps and network-controlled activation/deactivation	5267

A.6.6.22.2.1	Test purpose and Environment	5267

A.6.6.22.2.2	Test parameters	5267

A.6.6.22.2.3	Test Requirements	5269

A.6.6.23	SA event triggered reporting tests for concurrent measurement gaps with NCSG	5270

A.6.6.23.1	SA event triggered reporting tests for FR1 concurrent gaps with NCSG for partially partial overalpping scenario for SSB-based measurements in both inter-frequency layers [one MG + one NCSG]	5270

A.6.6.23.1.1	Test Purpose and Environment	5270

A.6.6.23.1.2	Test Requirements	5272

A.6.6.23.2	SA event triggered reporting tests for FR1 concurrent gaps with NCSG for partially partial overalpping scenario for SSB-based measurements in both inter-frequency layers [two NCSG]	5273

A.6.6.23.2.1	Test Purpose and Environment	5273

A.6.6.23.2.2	Test Requirements	5275

A.6.6.23.3	Event triggered reporting on SCC with deactivated SCell test with per-UE Con-NCSG under non-DRX	5275

A.6.6.23.3.1	Test purpose and Environment	5275

A.6.6.23.3.2	Test parameters	5276

A.6.6.23.3.3	Test Requirements	5278

A.6.6.24	SA event triggered reporting tests with NeedForGap in FR1	5278

A.6.6.24.1	SA event triggered reporting tests without gaps, with interruptions, under non-DRX	5278

A.6.6.24.1.1	Test purpose and Environment	5278

A.6.6.24.1.2	Test parameters	5278

A.6.6.24.1.3	Test Requirements	5280

A.6.6.24.2	SA event triggered reporting tests for FR1 without gap with interruption for inter-frequency measurement with SSB time index detection when DRX is not used	5280

A.6.6.24.2.1	Test Purpose and Environment	5280

A.6.6.24.2.2	Test parameters	5280

A.6.6.24.2.3	Test Requirements	5283

A.6.6.24.3	SA event triggered reporting tests for FR1 with ‘no-gap-with-interruption’, without measurement gap or DRX	5283

A.6.6.24.3.1	Test Purpose and Environment	5283

A.6.6.24.3.2	Test Requirements	5285

A.6.6.24.4	SA event triggered reporting tests for FR1 NeedForGaps without gap without interruption when DRX is not used	5285

A.6.6.24.4.1	Test Purpose and Environment	5285

A.6.6.24.4.2	Test parameters	5286

A.6.6.24.4.3	Test Requirements	5288

A.6.6.24.5	SA event triggered reporting tests without gap under non-DRX for UE indicating no-gap-no-interruption	5288

A.6.6.24.5.1	Test purpose and Environment	5288

A.6.6.24.5.2	Test parameters	5288

A.6.6.24.5.3	Test Requirements	5290

A.6.6.25	SA NR - E-UTRAN event-triggered without measurement gaps	5290

A.6.6.25.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	5290

A.6.6.25.1.1	Test Purpose and Environment	5290

A.6.6.25.1.2	Test Requirements	5293

A.6.6.25.2	SA NR - E-UTRAN event-triggered reporting without gap under non-DRX in FR1	5294

A.6.6.25.2.1	Test Purpose and Environment	5294

A.6.6.25.2.2	Test parameters	5294

A.6.6.25.2.3	Test Requirements	5295

A.6.6.25.3	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 for UE capable of inter-RAT EUTRAN measurement without gap when CRS is contained within UE’s active DL BWP	5295

A.6.6.25.3.1	Test Purpose and Environment	5295

A.6.6.25.3.2	Test Requirements	5298

A.6.6.26	LTM Intra-frequency L1-RSRP measurement	5299

A.6.6.26.1	Intra-frequency SSB based L1-RSRP measurement in FR1	5299

A.6.6.26.1.1	Test Purpose and Environment	5299

A.6.6.26.1.2	Test Parameters	5299

A.6.6.26.1.3	Test Requirements	5301

A.6.6.27	LTM Inter-frequency L1-RSRP measurement with measurement gap	5301

A.6.6.27.1	Inter-frequency SSB based L1-RSRP measurement with measurement gap	5301

A.6.6.27.1.1	Test Purpose and Environment	5301

A.6.6.27.1.2	Test parameters	5302

A.6.6.27.1.3	Test Requirements	5304

A.6.6.28	LTM Inter-frequency L1-RSRP measurement without measurement gap	5304

A.6.6.28.1	Inter-frequency SSB based L1-RSRP measurement without measurement gap	5304

A.6.6.28.1.1	Test Purpose and Environment	5304

A.6.6.28.1.2	Test parameters	5304

A.6.6.28.1.3	Test Requirements	5306

A.6.6.29	RSCPD Measurements	5307

A.6.6.29.1	NR RSCPD with RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_CONNECTED state	5307

A.6.6.29.1.1	Test Purpose and Environment	5307

A.6.6.29.1.2	Test Requirements	5314

A.6.6.30	RSCP Measurements	5314

A.6.6.30.1	DL RSCP with UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	5314

A.6.6.30.1.1	Test purpose and environment	5314

A.6.6.30.1.2	Test requirements	5318

A.6.7	Measurement Performance requirements	5318

A.6.7.1	SS-RSRP	5318

A.6.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	5318

A.6.7.1.1.1	Test Purpose and Environment	5318

A.6.7.1.1.2	Test parameters	5318

A.6.7.1.1.3	Test Requirements	5322

A.6.7.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	5322

A.6.7.1.2.1	Test Purpose and Environment	5322

A.6.7.1.2.2	Test parameters	5322

A.6.7.1.2.3	Test Requirements	5325

A.6.7.1.3	Void	5325

A.6.7.2	SS-RSRQ	5325

A.6.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	5325

A.6.7.2.1.1	Test Purpose and Environment	5325

A.6.7.2.1.2	Test Parameters	5326

A.6.7.2.1.3	Test Requirements	5329

A.6.7.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	5329

A.6.7.2.2.1	Test Purpose and Environment	5329

A.6.7.2.2.2	Test Parameters	5329

A.6.7.2.2.3	Test Requirements	5333

A.6.7.3	SS-SINR	5333

A.6.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	5333

A.6.7.3.1.1	Test Purpose and Environment	5333

A.6.7.3.1.2	Test Parameters	5334

A.6.7.3.1.3	Test Requirements	5337

A.6.7.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	5337

A.6.7.3.2.1	Test Purpose and Environment	5337

A.6.7.3.2.2	Test Parameters	5337

A.6.7.3.2.3	Test Requirements	5340

A.6.7.4	L1-RSRP measurement for beam reporting	5340

A.6.7.4.1	SSB based L1-RSRP measurement	5340

A.6.7.4.1.1	Test Purpose and Environment	5341

A.6.7.4.1.2	Test parameters	5341

A.6.7.4.1.3	Test Requirements	5344

A.6.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	5344

A.6.7.4.2.1	Test Purpose and Environment	5344

A.6.7.4.2.2	Test parameters	5344

A.6.7.4.2.3	Test Requirements	5347

A.6.7.5	E-UTRAN RSRP	5347

A.6.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell	5347

A.6.7.5.1.1	Test Purpose and Environment	5347

A.6.7.5.1.2	Test parameters	5347

A.6.7.5.1.3	Test Requirements	5350

A.6.7.6	E-UTRAN RSRQ	5350

A.6.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell	5350

A.6.7.6.1.1	Test Purpose and Environment	5350

A.6.7.6.1.2	Test parameters	5350

A.6.7.6.1.3	Test Requirements	5354

A.6.7.7	E-UTRAN RS-SINR	5354

A.6.7.7.1	SA: inter-RAT measurement accuracy with FR1 serving cell	5354

A.6.7.7.1.1	Test Purpose and Environment	5354

A.6.7.7.1.2	Test parameters	5354

A.6.7.7.1.3	Test Requirements	5357

A.6.7.8	CLI measurements	5357

A.6.7.8.1	SA SRS-RSRP measurement accuracy with FR1 serving cell	5357

A.6.7.8.1.1	Test Purpose and Environment	5357

A.6.7.8.1.2	Test parameters	5357

A.6.7.8.1.3	Test Requirements	5360

A.6.7.8.2	SA CLI-RSSI measurement accuracy with FR1 serving cell	5360

A.6.7.8.2.1	Test Purpose and Environment	5360

A.6.7.8.2.2	Test parameters	5361

A.6.7.8.2.3	Test Requirements	5362

A.6.7.9	L1-SINR measurement for beam reporting	5362

A.6.7.9.2	L1-SINR measurement with SSB based CMR and dedicated IMR	5365

A.6.7.9.2.1	Test Purpose and Environment	5366

A.6.7.9.2.2	Test parameters	5366

A.6.7.9.2.3	Test Requirements	5369

A.6.7.9.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	5369

A.6.7.9.3.1	Test Purpose and Environment	5369

A.6.7.9.3.2	Test parameters	5370

A.6.7.9.3.3	Test Requirements	5373

A.6.7.10	CSI-RSRP	5373

A.6.7.10.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	5373

A.6.7.10.1.1	Test Purpose and Environment	5373

A.6.7.10.1.2	Test parameters	5373

A.6.7.10.1.3	Test Requirements	5376

A.6.7.10.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	5376

A.6.7.10.2.1	Test Purpose and Environment	5377

A.6.7.10.2.2	Test parameters	5377

A.6.7.10.2.3	Test Requirements	5380

A.6.7.11	CSI-RSRQ	5380

A.6.7.11.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	5380

A.6.7.11.1.1	Test Purpose and Environment	5380

A.6.7.11.1.2	Test Parameters	5380

A.6.7.11.1.3	Test Requirements	5384

A.6.7.11.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	5384

A.6.7.11.2.1	Test Purpose and Environment	5384

A.6.7.11.2.2	Test Parameters	5384

A.6.7.11.2.3	Test Requirements	5388

A.6.7.12	CSI-SINR	5388

A.6.7.12.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	5388

A.6.7.12.1.1	Test Purpose and Environment	5388

A.6.7.12.1.2	Test Parameters	5388

A.6.7.12.1.3	Test Requirements	5391

A.6.7.12.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	5392

A.6.7.12.2.1	Test Purpose and Environment	5392

A.6.7.12.2.2	Test Parameters	5392

A.6.7.12.2.3	Test Requirements	5395

A.6.7.13	RSTD measurements	5395

A.6.7.13.1	RSTD measurement accuracy test case for single positioning frequency layer	5395

A.6.7.13.1.1	Test purpose and Environment	5395

A.6.7.13.1.2	Test Requirements	5397

A.6.7.13.2	RSTD measurement accuracy test case for dual positioning frequency layer	5397

A.6.7.13.2.1	Test purpose and Environment	5397

A.6.7.13.2.2	Test Requirements	5399

A.6.7.13.3	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_CONNECTED state	5399

A.6.7.13.3.1	Test purpose and Environment	5399

A.6.7.13.3.2	Test Requirements	5401

A.6.7.13.4	RSTD measurement accuracy test case with Rx TEG	5401

A.6.7.13.5	NR RSTD measurement accuracy test case for PRS aggregation in FR1 SA in RRC_CONNECTED mode	5403

A.6.7.13.5.1	Test purpose and Environment	5403

A.6.7.13.5.2	Test Requirements	5406

A.6.7.14	PRS-RSRP measurements	5406

A.6.7.14.1	SA: measurement accuracy with PRS in FR1	5406

A.6.7.14.1.1	Test Purpose and Environment	5406

A.6.7.14.1.2	Test parameters	5406

A.6.7.14.1.3	Test Requirements	5408

A.6.7.14.2	SA: measurement accuracy with PRS in FR1 with reduced sample number	5408

A.6.7.14.2.1	Test Purpose and Environment	5408

A.6.7.14.2.2	Test parameters	5408

A.6.7.14.2.3	Test Requirements	5410

A.6.7.14.3	Void	5410

A.6.7.14.3.1	Void	5410

A.6.7.14.3.2	Void	5410

A.6.7.14.3.3	Void	5410

A.6.7.15	UE Rx-Tx time difference measurements	5410

A.6.7.15.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA	5410

A.6.7.15.1.1	Test purpose and environment	5410

A.6.7.15.1.2	Test parameters	5411

A.6.7.15.1.3	Test requirements	5412

A.6.7.15.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR1 SA	5412

A.6.7.15.2.1	Test purpose and environment	5412

A.6.7.15.2.2	Test parameters	5413

A.6.7.15.2.3	Test requirements	5414

A.6.7.15.3	UE Rx-Tx time difference measurement accuracy with RxTx TEG	5414

A.6.7.15.3.1	Test purpose and environment	5414

A.6.7.15.3.2	Test parameters	5415

A.6.7.15.3.3	Test requirements	5417

A.6.7.15.4	UE Rx-Tx time difference measurement accuracy with PRS bandwidth aggregation in FR1 SA	5417

A.6.7.15.4.1	Test purpose and environment	5417

A.6.7.15.4.2	Test requirements	5420

A.6.7.16	PRS-RSRPP measurements	5420

A.6.7.16.1	SA: measurement accuracy with PRS in FR1	5420

A.6.7.16.1.1	Test Purpose and Environment	5420

A.6.7.16.1.2	Test parameters	5420

A.6.7.16.1.3	Test Requirements	5422

A.6.7.16.2	SA: measurement accuracy with reduced PRS samples in FR1	5422

A.6.7.16.2.1	Test Purpose and Environment	5422

A.6.7.16.2.2	Test parameters	5422

A.6.7.17	LTM L1-RSRP measurement	5424

A.6.7.17.1	Inter-frequency L1-RSRP accuracy requirements for neighbour cell in FR1	5424

A.6.7.17.1.1	Test Purpose and Environment	5424

A.6.7.17.1.2	Test parameters	5425

A.6.7.17.1.3	Test Requirements	5428

A.6.7.18	TDCP amplitude measurement accuracy	5428

A.6.7.18.1	TDCP amplitude measurement accuracy in FR1	5428

A.6.7.18.1.1	Test Purpose and Environment	5428

A.6.7.18.1.2	Test parameters	5428

A.6.7.18.1.3	Test Requirements	5429

A.6.7.19	RSCPD Measurements	5430

A.6.7.19.1	RSCPD with RSTD measurement accuracy in FR1 SA in RRC_CONNECTED	5430

A.6.7.19.1.1	Test purpose and environment	5430

A.6.7.19.1.2	Test parameters	5430

A.6.7.19.1.3	Test requirements	5433

A.6.7.20	RSCP Measurements	5433

A.6.7.20.1	RSCP with UE Rx-Tx time difference measurement accuracy in FR1 SA	5433

A.6.7.20.1.1	Test purpose and environment	5433

A.6.7.20.1.2	Test parameters	5434

A.6.7.20.1.3	Test requirements	5437

A.6.8	Measurement procedure in RRC_INACTIVE	5437

A.6.8.1	RSTD measurements	5437

A.6.8.1.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_INACTIVE state	5437

A.6.8.1.1.1	Test Purpose and Environment	5437

A.6.8.1.1.2	Test Requirements	5440

A.6.8.1.2	NR RSTD measurement reporting delay test case with reduced number of samples in RRC_INACTIVE, FR1 SA	5440

A.6.8.1.2.1	Test Purpose and Environment	5440

A.6.8.1.2.1	Test Purpose and Environment	5440

A.6.8.1.2.2	Test Requirements	5443

A.6.8.1.3	NR RSTD measurement reporting delay test case for PRS aggregation in FR1 SA in RRC_INACTIVE state	5444

A.6.8.1.3.1	Test purpose and environment	5444

A.6.8.1.3.2	Test requirements	5447

A.6.8.1.4	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_INACTIVE state when eDRX cycle > 10.24s for non-RedCap UE	5448

A.6.8.1.4.1	Test Purpose and Environment	5448

A.6.8.1.4.2	Test Requirements	5451

A.6.8.2	PRS-RSRP measurements	5451

A.6.8.2.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_INACTIVE	5451

A.6.8.2.1.1	Test purpose and Environment	5451

A.6.8.2.1.2	Test Requirements	5453

A.6.8.2.2	PRS-RSRP reporting delay test case with reduced number of samples in RRC_INACTIVE	5453

A.6.8.2.2.1	Test purpose and Environment	5453

A.6.8.2.2.2	Test Requirements	5455

A.6.8.2.3	PRS-RSRP reporting delay test case in RRC_INACTIVE state in FR1 with eDRX cycle > 10.24s	5456

A.6.8.2.3.1	Test purpose and Environment	5456

A.6.8.2.3.2	Test Requirements	5458

A.6.8.3	UE Rx-Tx time difference measurements	5459

A.6.8.3.1	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	5459

A.6.8.3.1.1	Test purpose and environment	5459

A.6.8.3.1.2	Test requirements	5461

A.6.8.3.2	UE Rx-Tx time difference measurement with reduced number of samples in RRC_INACTIVE, FR1 SA	5461

A.6.8.3.2.1	Test purpose and environment	5461

A.6.8.3.2.2	Test requirements	5463

A.6.8.3.3	UE Rx-Tx time difference measurement for single positioning frequency layer with eDRX > 10.24s in FR1 SA	5464

A.6.8.3.3.1	Test purpose and environment	5464

A.6.8.3.3.2	Test requirements	5468

A.6.8.3.4	UE Rx-Tx time difference measurements with PRS bandwidth aggregation in FR1 SA	5468

A.6.8.3.4.1	Test purpose and environment	5468

A.6.8.3.4.2	Test requirements	5471

A.6.8.4	PRS-RSRPP measurements	5471

A.6.8.4.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_INACTIVE state	5471

A.6.8.4.1.1	Test purpose and Environment	5472

A.6.8.4.1.2	Test Requirements	5473

A.6.8.4.2	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_INACTIVE state for reduced number of samples	5474

A.6.8.4.2.1	Test purpose and Environment	5474

A.6.8.4.2.2	Test Requirements	5476

A.6.8.4.3	PRS-RSRPP reporting delay in RRC_INACTIVE with eDRX	5476

A.6.8.4.3.1	Test purpose and Environment	5476

A.6.8.4.3.2	Test Requirements	5479

A.6.8.5	RSCPD Measurements	5479

A.6.8.5.1	DL RSCPD reported with RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_INACTIVE state	5479

A.6.8.5.1.1	Test Purpose and Environment	5479

A.6.8.5.1.2	Test Requirements	5479

A.6.8.6	RSCP Measurements	5480

A.6.8.6.1	DL RSCP with UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	5480

A.6.8.6.1.1	Test purpose and environment	5480

A.6.8.6.1.2	Test requirements	5484

A.6.9	Measurement performance requirements in RRC_INACTIVE	5484

A.6.9.1	RSTD measurements	5484

A.6.9.1.1	RSTD measurement accuracy test case for single positioning frequency layer in FR1 in RRC_INACTIVE state	5484

A.6.9.1.1.1	Test purpose and Environment	5484

A.6.9.1.1.2	Test Requirements	5486

A.6.9.1.2	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_INACTIVE state	5486

A.6.9.1.2.1	Test purpose and Environment	5486

A.6.9.1.2.2	Test Requirements	5488

A.6.9.1.3	RSTD measurement accuracy for PRS aggregation in FR1 in RRC_INACTIVE state	5488

A.6.9.1.3.1	Test purpose and Environment	5488

A.6.9.1.3.2	Test Requirements	5491

A.6.9.2	PRS-RSRP measurements	5491

A.6.9.2.1	SA: measurement accuracy with PRS in FR1 in RRC_INACTIVE	5491

A.6.9.2.1.1	Test Purpose and Environment	5491

A.6.9.2.1.2	Test parameters	5491

A.6.9.2.1.3	Test Requirements	5493

A.6.9.2.2	SA: measurement accuracy with PRS in FR1 with reduced number of samples in RRC_INACTIVE state	5493

A.6.9.2.2.1	Test Purpose and Environment	5493

A.6.9.2.2.2	Test parameters	5493

A.6.9.2.2.3	Test Requirements	5495

A.6.9.3	UE Rx-Tx time difference measurements	5495

A.6.9.3.1.1	UE Rx-Tx time difference measurement accuracy in FR1 SA	5495

A.6.9.3.1.1.1	Test purpose and environment	5495

A.6.9.3.1.1.2 Test parameters	5496

A.6.9.3.1.1.3	Test requirements	5497

A.6.9.3.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples	5497

A.6.9.3.2.1	Test purpose and environment	5497

A.6.9.3.2.2	Test parameters	5497

A.6.9.3.2.3	Test requirements	5499

A.6.9.3.3	UE Rx-Tx time difference measurement accuracy with PRS bandwidth aggregation in FR1 SA	5499

A.6.9.3.3.1	Test purpose and environment	5499

A.6.9.3.3.2	Test requirements	5502

A.6.9.4	PRS-RSRPP measurements	5502

A.6.9.4.1	SA: PRS-RSRPP measurement accuracy in FR1 in RRC INACTIVE	5502

A.6.9.4.1.1	Test Purpose and Environment	5502

A.6.9.4.1.2	Test parameters	5502

A.6.9.4.1.3	Test Requirements	5504

A.6.9.4.2	SA: measurement accuracy with reduced PRS samples in FR1 in RRC INACTIVE	5504

A.6.9.4.2.1	Test Purpose and Environment	5504

A.6.9.4.2.2	Test parameters	5505

A.6.9.4.2.3	Test Requirements	5506

A.6.9.5	RSCPD Measurements	5507

A.6.9.5.1	RSCPD with RSTD measurement accuracy in FR1 SA in RRC_INACTIVE	5507

A.6.9.5.1.1	Test purpose and environment	5507

A.6.9.5.1.2	Test parameters	5507

A.6.9.5.1.3	Test requirements	5510

A.6.9.6	RSCP Measurements	5510

A.6.9.6.1	RSCP with UE Rx-Tx time difference measurement accuracy in FR1 SA	5510

A.6.9.6.1.1	Test purpose and environment	5510

A.6.9.6.1.2	Test parameters	5511

A.6.9.6.1.3	Test requirements	5514

A.6.10	Measurement Procedure in RRC_IDLE	5514

A.6.10.1	RSTD Measurements	5514

A.6.10.1.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_IDLE state for non-RedCap UE	5514

A.6.10.1.1.1	Test purpose and environment	5514

A.6.10.1.1.2	Test requirements	5517

A.6.10.1.2	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_IDLE state with eDRX cycle > 10.24s for non-RedCap UE	5518

A.6.10.1.2.1	Test Purpose and Environment	5518

A.6.10.1.2.2	Test Requirements	5521

A.6.10.1.3	NR RSTD measurement reporting delay test case for PRS aggregation in FR1 SA in RRC_IDLE state	5521

A.6.10.1.3.1	Test purpose and environment	5521

A.6.10.1.3.2	Test requirements	5521

A.6.10.2	 PRS-RSRP Measurements	5522

A.6.10.2.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_IDLE state for non-RedCap UE in FR1	5522

A.6.10.2.1.1	Test purpose and Environment	5522

A.6.10.2.1.2	Test Requirements	5524

A.6.10.2.2	PRS-RSRP reporting delay test case in RRC_IDLE state in FR1 when eDRX cycle > 10.24s	5525

A.6.10.2.2.1	Test purpose and Environment	5525

A.6.10.2.2.2	Test Requirements	5525

A.6.10.3	RSCPD Measurements	5525

A.6.10.3.1	DL RSCPD reported with RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_IDLE state	5525

A.6.10.3.1.1	Test Purpose and Environment	5525

A.6.10.3.1.2	Test Requirements	5526

A.6.11	Measurement Performance Requirements in RRC_IDLE	5526

A.6.11.1	RSTD Measurements	5526

A.6.11.1.1	NR RSTD measurement accuracy test case for single positioning frequency layer in FR1 SA in RRC_IDLE state for non-RedCap UE	5526

A.6.11.1.1.1	Test purpose and environment	5526

A.6.11.1.1.2	Test requirements	5528

A.6.11.1.2	RSTD measurement accuracy test case for single positioning frequency layer in FR1 in RRC_IDLE state with eDRX>10.24s for non-RedCap UE	5528

A.6.11.1.2.1	Test purpose and Environment	5528

A.6.11.1.2.2	Test Requirements	5530

A.6.11.1.3	NR RSTD measurement accuracy test case for PRS aggregation in FR1 SA in RRC_IDLE state	5530

A.6.11.1.3.1	Test purpose and environment	5530

A.6.11.1.3.2	Test requirements	5530

A.6.11.2	PRS-RSRP measurements	5530

A.6.11.2.1	PRS-RSRP measurement accuracy test case for non-RedCap UE in FR1 in RRC_IDLE state	5530

A.6.11.2.1.1	Test Purpose and Environment	5530

A.6.11.2.1.2	Test parameters	5531

A.6.11.2.1.3	Test Requirements	5533

A.6.11.2.2	PRS-RSRP measurement accuracy test case in RRC_IDLE state in FR1 when eDRX cycle > 10.24s	5533

A.6.11.2.2.1	Test purpose and Environment	5533

A.6.11.2.2.2	Test parameters	5533

A.6.11.2.2.3	Test Requirements	5534

A.6.11.3	RSCPD Measurements	5534

A.6.11.3.1	RSCPD with RSTD measurement accuracy in FR1 SA in RRC_IDLE	5534

A.6.11.3.1.1	Test purpose and environment	5534

A.6.11.3.1.2	Test parameters	5534

A.6. 11.3.1.3	Test requirements	5537

A.7.5	Signaling characteristics	5538

A.7.5.1	Radio link Monitoring	5538

A.7.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	5538

A.7.5.1.1.1	Test Purpose and Environment	5538

A.7.5.1.1.2	Test Requirements	5541

A.7.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	5541

A.7.5.1.2.1	Test Purpose and Environment	5541

A.7.5.1.2.2	Test Requirements	5544

A.7.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	5544

A.7.5.1.3.1	Test Purpose and Environment	5544

A.7.5.1.3.2	Test Requirements	5546

A.7.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	5547

A.7.5.1.4.1	Test Purpose and Environment	5547

A.7.5.1.4.2	Test Requirements	5549

A.7.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	5549

A.7.5.1.5.1	Test Purpose and Environment	5549

A.7.5.1.5.2	Test Requirements	5553

A.7.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	5553

A.7.5.1.6.1	Test Purpose and Environment	5553

A.7.5.1.6.2	Test Requirements	5557

A.7.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	5557

A.7.5.1.7.1	Test Purpose and Environment	5557

A.7.5.1.7.2	Test Requirements	5560

A.7.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	5560

A.7.5.1.8.1	Test Purpose and Environment	5560

A.7.5.1.8.2	Test Requirements	5564

A.7.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	5564

A.7.5.1.9.1	Test Purpose and Environment	5564

A.7.5.1.9.2	Test Requirements	5566

A.7.5.1.10	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode for UE supporting fast beam sweeping in multi-Rx	5566

A.7.5.1.10.1	Test Purpose and Environment	5566

A.7.5.1.10.2	Test Requirements	5569

A.7.5.1.11	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode when CD-SSB is outside active BWP	5569

A.7.5.1.11.1	Test Purpose and Environment	5569

A.7.5.1.12	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode when CD-SSB is outside active BWP	5569

A.7.5.1.12.1	Test Purpose and Environment	5569

A.7.5.1.12.2	Test Requirements	5570

A.7.5.1.13	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode for UE supporting NCD-SSB based measurement outside active BWP	5570

A.7.5.1.13.1	Test Purpose and Environment	5570

A.7.5.1.13.2	Test Requirements	5573

A.7.5.2	Interruption	5573

A.7.5.2.1	Interruptions during measurements on deactivated NR SCC in FR2	5573

A.7.5.2.1.1	Test Purpose and Environment	5573

A.7.5.2.1.2	Test Requirements	5575

A.7.5.2.2	SA interruptions at NR SRS carrier-based switching	5576

A.7.5.2.2.1	Test Purpose and Environment	5576

A.7.5.2.2.2	Test Parameters	5576

A.7.5.2.2.3	Test Requirements	5577

A.7.5.3	SCell Activation and Deactivation Delay	5577

A.7.5.3.1	SCell Activation and deactivation for SCell in FR2 intra-band in non-DRX	5577

A.7.5.3.1.1	Test Purpose and Environment	5578

A.7.5.3.1.2	Test Requirements	5579

A.7.5.3.2	SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2	5579

A.7.5.3.2.1	Test Purpose and Environment	5579

A.7.5.3.2.2	Test Requirements	5582

A.7.5.3.3	SCell Activation and deactivation for SCell in FR2 inter-band in non-DRX	5582

A.7.5.3.3.1	Test Purpose and Environment	5582

A.7.5.3.3.2	Test Requirements	5585

A.7.5.3.4	Direct SCell activation at SCell addition of known SCell in FR2	5585

A.7.5.3.4.1	Test Purpose and Environment	5585

A.7.5.3.4.2	Test Requirements	5587

A.7.5.3.5	Direct SCell activation at handover with known SCell in FR2	5588

A.7.5.3.5.1	Test Purpose and Environment	5588

A.7.5.3.5.2	Test Requirements	5590

A.7.5.3.6	PUCCH SCell activation and deactivation for FR1+FR2 inter-band with target SCell in FR2 and known	5591

A.7.5.3.6.1	Test Purpose and Environment	5591

A.7.5.3.6.2	Test Requirements	5593

A.7.5.3.7	PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR1 PCell	5594

A.7.5.3.7.1	Test Purpose and Environment	5594

A.7.5.3.7.2	Test Requirements	5597

A.7.5.3.8	SCell Activation and deactivation for known PUCCH SCell in FR2 inter-band in non-DRX	5598

A.7.5.3.8.1	Test Purpose and Environment	5598

A.7.5.3.8.2	Test Requirements	5601

A.7.5.3.9	PUCCH SCell Activation and deactivation of unknown SCell in FR2	5602

A.7.5.3.9.1	Test Purpose and Environment	5602

A.7.5.3.9.2	Test Requirements	5604

A.7.5.3.10	SCell Activation and deactivation of FR2 known PUCCH SCell and one FR2 unknown SCell with FR2 PCell	5605

A.7.5.3.10.1	Test Purpose and Environment	5605

A.7.5.3.10.2	Test Requirements	5608

A.7.5.3.11	PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR2 PCell	5609

A.7.5.3.11.1	PUCCH SCell activation with non-PUCCH SCell in a secondary PUCCH Group	5609

A.7.5.3.11.1.1	Test Purpose and Environment	5609

A.7.5.3.11.1.2	Test Requirements	5612

A.7.5.3.11.2	PUCCH SCell activation with non-PUCCH SCell in a primary PUCCH Group	5613

A.7.5.3.11.2.1	Test Purpose and Environment	5613

A.7.5.3.11.2.2	Test Requirements	5616

A.7.5.3.12	Void	5617

A.7.5.3.13	SCell Activation for SCell in FR2 intra-band in non-DRX	5617

A.7.5.3.13.1	Test Purpose and Environment	5617

A.7.5.3.13.2	Test Requirements	5619

A.7.5.3.14	SCell Activation for known SCell in FR2 inter-band	5619

A.7.5.3.14.1	Test Purpose and Environment	5619

A.7.5.3.14.2	Test Requirements	5621

A.7.5.3.15	PUCCH SCell activation and deactivation with FR1 PCell based on L3 reporting after SCell activation command	5622

A.7.5.3.15.1	Test Purpose and Environment	5622

A.7.5.3.15.2	Test Requirements	5626

A.7.5.3.16	PUCCH SCell activation and deactivation with FR2 PCell based on L3 reporting after SCell activation command	5626

A.7.5.3.16.1	Test Purpose and Environment	5626

A.7.5.3.16.2	Test Requirements	5629

A.7.5.3.17	SCell Activation and deactivation for SCell in FR2 inter-band in DRX for UE capable of small beam sweeping factors and/or short measurement interval	5630

A.7.5.3.17.1	Test Purpose and Environment	5630

A.7.5.3.17.2	Test Requirements	5632

A.7.5.3.18	SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2, in DRX, for UE capable of small beam sweeping factors and/or short measurement interval	5634

A.7.5.3.18.1	Test Purpose and Environment	5634

A.7.5.3.18.2	Test Requirements	5637

A.7.5.3.19	SCell Activation and deactivation of FR2 unknown SCell with FR1 PCell in non-DRX with L3 reporting during activation	5639

A.7.5.3.19.1	Test Purpose and Environment	5639

A.7.5.3.19.2	Test Requirements	5642

A.7.5.3.20	SCell Activation and Deactivation of FR2 unkown SCell with FR2 PCell in non-DRX with L3 reporting during activation	5642

A.7.5.3.20.1	Test Purpose and Environment	5643

A.7.5.3.20.2	Test Requirements	5645

A.7.5.4	Void	5646

A.7.5.5	Beam Failure Detection and Link recovery procedures	5646

A.7.5.5.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	5646

A.7.5.5.1.1	Test Purpose and Environment	5646

A.7.5.5.1.2	Test Requirements	5649

A.7.5.5.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	5650

A.7.5.5.2.1	Test Purpose and Environment	5650

A.7.5.5.2.2	Test Requirements	5653

A.7.5.5.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	5653

A.7.5.5.3.1	Test Purpose and Environment	5653

A.7.5.5.3.2	Test Requirements	5656

A.7.5.5.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	5656

A.7.5.5.4.1	Test Purpose and Environment	5656

A.7.5.5.4.2	Test Requirements	5659

A.7.5.5.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	5660

A.7.5.5.5.1	Test Purpose and Environment	5660

A.7.5.5.5.2	Test Requirements	5663

A.7.5.5.6	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	5663

A.7.5.5.6.1	Test Purpose and Environment	5663

A.7.5.5.6.2	Test Requirements	5666

A.7.5.5.7	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	5666

A.7.5.5.7.1	Test Purpose and Environment	5666

A.7.5.5.7.2	Test Requirements	5669

A.7.5.5.8	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode for UE fulfilling relaxed measurement criterion	5670

A.7.5.5.8.1	Test Purpose and Environment	5670

A.7.5.5.8.2	Test Requirements	5673

A.7.5.5.9	TRP specific Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	5673

A.7.5.5.9.1	Test Purpose and Environment	5673

A.7.5.5.9.2	Test Requirements	5676

A.7.5.5.10	TRP specific Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	5677

A.7.5.5.10.1	Test Purpose and Environment	5677

A.7.5.5.10.2	Test Requirements	5680

A.7.5.5.11	Beam Failure Detection and Link Recovery Test for FR2-2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	5680

A.7.5.5.11.1	Test Purpose and Environment	5680

A.7.5.5.11.2	Test Requirements	5683

A.7.5.5.12	Beam Failure Detection and Link Recovery Test for FR2-2 PCell configured with CSI-RS-based BFD and LR in DRX mode	5683

A.7.5.5.12.1	Test Purpose and Environment	5683

A.7.5.5.12.2	Test Requirements	5686

A.7.5.5.13	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2-2 PCell configured with SSB-based BFD and LR in non-DRX mode	5687

A.7.5.5.13.1	Test Purpose and Environment	5687

A.7.5.5.13.2	Test Requirements	5689

A.7.5.5.14	TRP specific Beam Failure Detection and Link Recovery for FR2 PCell configured with CSI-RS-based BFD and LR and multi-Rx operation in DRX mode	5689

A.7.5.5.14.1	Test Purpose and Environment	5689

A.7.5.5.14.2	Test Requirements	5693

A.7.5.6	Active BWP switch	5693

A.7.5.6.1	DCI-based and Timer-based Active BWP Switch	5693

A.7.5.6.1.1	NR FR2- NR FR2 DL active BWP switch of SCell with non-DRX in SA	5693

A.7.5.6.1.2	NR FR1- NR FR2 DL active BWP switch of SCell with non-DRX in SA	5697

A.7.5.6.1.3	NR FR2 DL active BWP switch with non-DRX in SA	5701

A.7.5.6.1.3.1	Test Purpose and Environment	5701

A.7.5.6.1.3.2	Test Requirements	5703

A.7.5.6.1.4	NR FR2-2- NR FR2-2 DL active BWP switch of SCell with non-DRX in SA	5703

A.7.5.6.1.4.1	Test Purpose and Environment	5703

A.7.5.6.1.4.2	Test Requirements	5706

A.7.5.6.2	RRC-based Active BWP Switch	5707

A.7.5.6.2.1.1	Test Purpose and Environment	5707

A.7.5.6.2.1.2	Test Requirements	5710

A.7.5.6.2.2	NR FR2-2 DL active BWP switch of PCell with non-DRX in SA	5710

A.7.5.6.2.2.1	Test Purpose and Environment	5710

A.7.5.6.2.2.2	Test Requirements	5712

A.7.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	5713

A.7.5.6.3.1.1	Test Purpose and Environment	5713

A.7.5.6.3.1.2	Test Requirements	5715

A.7.5.6.4	SCell dormancy switch	5716

A.7.5.6.4.1	NR FR2 PCell SCell dormancy switch of single FR2 SCell inside active time	5716

A.7.5.6.4.1.1	Test Purpose and Environment	5716

A.7.5.6.4.1.2	Test Requirements	5718

A.7.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR2 SCells outside active time	5719

A.7.5.6.4.2.1	 Test Purpose and Environment	5719

A.7.5.6.4.2.2	 Test Requirements	5722

A.7.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	5722

A.7.5.6.5.1	Active BWP switch on multiple SCells with non-DRX in SA	5722

A.7.5.6.5.2	NR FR2-2 Active BWP switch on multiple SCells with non-DRX in SA	5724

A.7.5.6.5.2.1	Test Purpose and Environment	5724

A.7.5.6.5.2.2	Test Requirements	5727

A.7.5.7	PSCell addition and release delay	5727

A.7.5.7.1	Addition and Release Delay of known NR PSCell	5727

A.7.5.7.1.1	Test Purpose and Environment	5727

A.7.5.7.1.2	Test Requirements	5730

A.7.5.7.2	Addition and Release Delay of unknown NR PSCell in	5730

A.7.5.7.2.1	Test Purpose and Environment	5730

A.7.5.7.2.2	Test Requirements	5732

A.7.5.7.3	Addition and Release Delay of known NR PSCell in FR2-2	5732

A.7.5.7.3.1	Test Purpose and Environment	5732

A.7.5.7.3.2	Test Requirements	5735

A.7.5.7.4	Addition and Release Delay of unknown NR PSCell in FR2-2	5735

A.7.5.7.4.1	Test Purpose and Environment	5735

A.7.5.7.4.2	Test Requirements	5737

A.7.5.8	Active TCI state switch delay	5738

A.7.5.8.1	MAC-CE based active TCI state switch	5738

A.7.5.8.2	RRC based active TCI state switch	5741

A.7.5.8.3	MAC-CE based active TCI state switch for HST FR2 scenario	5744

A.7.5.8.3.1	NR PCell FR2 HST active TCI state switch for a known TCI state	5744

A.7.5.8.3.1.1	Test Purpose and Environment	5744

A.7.5.8.3.1.2	Test Requirements	5747

A.7.5.8.3.2	NR PCell FR2 HST active TCI state switch for PC6 UE supporting tci-StateSwitchInd-r18 for a known TCI state	5748

A.7.5.8.3.2.1	Test Purpose and Environment	5748

A.7.5.8.3.2.2	Test Requirements	5751

A.7.5.8.4	DCI based active TCI state switch with m-DCI for simultaneous reception	5751

A.7.5.8.4.1	Test Purpose and Environment	5751

A.7.5.8.4.2	Test Requirements	5754

A.7.5.8.5	Single-DCI FR2 DCI based active TCI state switch with known target TCI states for simultaneous reception	5754

A.7.5.8.5.1	Test Purpose and Environment	5754

A.7.5.8.5.1.2	Test Requirements	5756

A.7.5.9	Uplink spatial relation switch delay	5757

A.7.5.9.1.1.1	Test Purpose and Environment	5757

A.7.5.9.1.1.2	Test Requirements	5759

A.7.5.9.2	RRC based spatial relation switch	5759

A.7.5.9.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	5759

A.7.5.9.2.1.1	Test Purpose and Environment	5759

A.7.5.9.2.1.2	Test Requirements	5761

A.7.5.10	UE specific CBW change	5761

A.7.5.10.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	5761

A.7.5.10.1.1	Test Purpose and Environment	5761

A.7.5.10.1.2	Test Requirements	5763

A.7.5.11	UE UL carrier RRC reconfiguration Delay	5764

A.7.5.11.1	UE UL carrier RRC reconfiguration Delay	5764

A.7.5.11.1.1	Test Purpose and Environment	5764

A.7.5.11.1.2	Test Requirements	5766

A.7.5.12	Conditional PSCell addition and release delay (FR2 SA)	5766

A.7.5.12.1	Addition and Release Delay of PSCell	5766

A.7.5.12.1.1	Test purpose and environment	5766

A.7.5.12.1.2	Test Parameters	5766

A.7.5.12.1.3	Test Requirements	5768

A.7.5.13	Unified TCI state switching delay	5768

A.7.5.13.1	MAC-CE based active joint TCI state switching	5768

A.7.5.13.1.1	NR PCell FR2 active joint TCI state switch for a known TCI state	5768

A.7.5.13.1.1.1	Test Purpose and Environment	5768

A.7.5.13.1.1.2	Test parameters	5769

A.7.5.13.1.1.3	Test Requirements	5770

A.7.5.13.2	 MAC-CE based active uplink TCI state switch	5771

A.7.5.13.2.1	 NR FR2 PCell uplink TCI state switch for a known TCI state	5771

A.7.5.13.2.1.1	Test Purpose and Environment	5771

A.7.5.13.2.1.2	Test parameters	5771

A.7.5.13.2.1.3	Test Requirements	5773

A.7.5.13.3	MAC-CE based active downlink TCI state switch	5773

A.7.5.13.3.1	NR PCell FR2 active downlink TCI state switch to cell with additional PCI for a known TCI state	5773

A.7.5.13.3.1.1	Test Purpose and Environment	5773

A.7.5.13.3.1.2	Test Parameters	5773

A.7.5.13.3.1.3	Test Requirements	5776

A.7.5.13.4	sDCI MAC-CE based joint TCI state switching	5777

A.7.5.13.4.1	NR PCell FR2 dual downlink and uplink TCI state switch in sDCI for known case	5777

A.7.5.13.4.1.1	Test Purpose and Environment	5777

A.7.5.13.4.1.2	Test parameters	5777

A.7.5.13.4.1.3	Test Requirements	5779

A.7.5.13.5	MAC-CE based dual downlink TCI state switching delay for unified TCI for single-DCI mTRP	5779

A.7.5.13.5.1	NR PCell FR2 dual downlink TCI state switch in sDCI for known case	5779

A.7.5.13.5.1.1	Test Purpose and Environment	5779

A.7.5.13.5.1.2	Test Parameters	5780

A.7.5.13.5.1.3	Test Requirements	5782

A.7.5.13.6	 MAC-CE based active uplink TCI state switch for single-DCI mTRP	5782

A.7.5.13.6.1	 NR FR2 PCell uplink TCI state switch for two known TCI states	5782

A.7.5.13.6.1.1	Test Purpose and Environment	5782

A.7.5.13.6.1.2	Test parameters	5783

A.7.5.13.6.1.3	Test Requirements	5784

A.7.5.14	PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2	5784

A.7.5.14.1	Test Purpose and Environment	5784

A.7.5.14.2	Test Requirements	5787

A.7.5.15	Void	5787

A.7.5.16	UE L1-RSRP Scheduling and Measurement Restrictions on FR2-1	5788

A.7.5.16.1	Test Purpose and Environment	5788

A.7.5.16.2	Test Requirements	5790

A.7.5.17	SCG Activation and deactivation for FR1+FR1 inter-band with target PSCell in FR1	5791

A.7.5.17.1	Test Purpose and Environment	5791

A.7.5.17.2	Test Requirements	5793

A.7.5.18	Subsequent conditional PSCell addition/change	5794

A.7.5.18.1	Intra-frequency subsequent CPC from FR1-FR2 NR-DC to FR1-FR2 NR-DC	5794

A.7.5.18.1.1	Test purpose and environment	5794

A.7.5.18.1.2	Test Requirements	5797

A.7.5.18.2	Inter-frequency subsequent CPA from FR1-FR2 NR-DC to FR1-FR2 NR-DC	5798

A.7.5.18.2.1	Test Purpose and Environment	5798

A.7.5.18.2.2	Test Requirements	5800

A.7.6	Measurement procedure	5803

A.7.6.1	Intra-frequency Measurements	5803

A.7.6.1.1	SA event triggered reporting test without gap under non-DRX	5803

A.7.6.1.1.1	Test purpose and Environment	5803

A.7.6.1.1.2	Test Requirements	5805

A.7.6.1.2	SA event triggered reporting test without gap under DRX	5805

A.7.6.1.2.1	Test purpose and Environment	5805

A.7.6.1.2.2	Test Requirements	5807

A.7.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	5807

A.7.6.1.3.1	Test purpose and Environment	5807

A.7.6.1.3.2	Test Requirements	5810

A.7.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	5810

A.7.6.1.4.1	Test purpose and Environment	5810

A.7.6.1.4.2	Test Requirements	5812

A.7.6.1.5	SA event triggered reporting test without gap under non-DRX for UE configured with highSpeedMeasFlagFR2-r17	5813

A.7.6.1.5.1	Test purpose and Environment	5813

A.7.6.1.5.2	Test Requirements	5815

A.7.6.1.6	SA event triggered reporting test without gap under non-DRX for FR2-2	5815

A.7.6.1.6.1	Test purpose and Environment	5815

A.7.6.1.6.2	Test Requirements	5817

A.7.6.1.7	SA event triggered reporting test without gap under DRX for FR2-2	5818

A.7.6.1.7.1	Test purpose and Environment	5818

A.7.6.1.7.2	Test Requirements	5820

A.7.6.1.8	SA event triggered reporting test with per-UE gaps under non-DRX for FR2-2	5821

A.7.6.1.8.1	Test purpose and Environment	5821

A.7.6.1.8.2	Test Requirements	5823

A.7.6.1.9	SA event triggered reporting test with per-UE gaps under DRX for FR2-2	5824

A.7.6.1.9.1	Test purpose and Environment	5824

A.7.6.1.9.2	Test Requirements	5826

A.7.6.1.10	SA event triggered reporting test with SSB time index detection without gap under non-DRX for FR2-2	5827

A.7.6.1.10.1	Test purpose and Environment	5827

A.7.6.1.10.2	Test Requirements	5829

A.7.6.1.11	SA event triggered reporting test with SSB time index detection with per-UE gaps under non-DRX for FR2-2	5829

A.7.6.1.11.1	Test purpose and Environment	5829

A.7.6.1.11.2	Test Requirements	5831

A.7.6.1.12	SA event triggered reporting test without gap under non-DRX when CD-SSB is outside active BWP	5832

A.7.6.1.12.1	Test purpose and Environment	5832

A.7.6.1.12.2	Test Requirements	5832

A.7.6.1.13	SA event triggered reporting test without gap under non-DRX with NCD-SSB	5832

A.7.6.1.13.1	Test purpose and Environment	5832

A.7.6.1.13.2	Test Requirements	5834

A.7.6.1.14	SA event triggered reporting test without gap under non-DRX for power class 6 UE supporting measEnhCAInterFreqFR2-r18	5835

A.7.6.1.14.1	Test Purpose and Environment	5835

A.7.6.1.14.2	Test Requirements	5836

A.7.6.2	Inter-frequency Measurements	5836

A.7.6.2.1	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR2)	5836

A.7.6.2.1.1	Test Purpose and Environment	5836

A.7.6.2.1.2	Test Requirements	5838

A.7.6.2.2	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (PCell in FR2)	5839

A.7.6.2.2.1	Test Purpose and Environment	5839

A.7.6.2.2.2	Test Requirements	5841

A.7.6.2.3	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR2)	5841

A.7.6.2.3.1	Test Purpose and Environment	5841

A.7.6.2.3.2	Test Requirements	5843

A.7.6.2.4	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR2)	5844

A.7.6.2.4.1	Test Purpose and Environment	5844

A.7.6.2.4.2	Test Requirements	5846

A.7.6.2.5	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1)	5846

A.7.6.2.5.1	Test Purpose and Environment	5846

A.7.6.2.5.2	Test Requirements	5849

A.7.6.2.6	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (PCell in FR1)	5849

A.7.6.2.6.1	Test Purpose and Environment	5849

A.7.6.2.6.2	Test Requirements	5852

A.7.6.2.7	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1)	5852

A.7.6.2.7.1	Test Purpose and Environment	5852

A.7.6.2.7.2	Test Requirements	5855

A.7.6.2.8	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1)	5855

A.7.6.2.8.1	Test Purpose and Environment	5855

A.7.6.2.8.2	Test Requirements	5858

A.7.6.2.9	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2) (rel16 additional mandatory gap pattern 17)	5858

A.7.6.2.9.1	Test Purpose and Environment	5858

A.7.6.2.9.2	Test Requirements	5860

A.7.6.2.10	SA event triggered reporting test without gap under non-DRX	5861

A.7.6.2.10.1	Test Purpose and Environment	5861

A.7.6.2.10.2	Test Requirements	5862

A.7.6.2.11	SA event triggered reporting test without gap under DRX	5863

A.7.6.2.11.1	Test Purpose and Environment	5863

A.7.6.2.11.2	Test Requirements	5864

A.7.6.2.12	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is not used (PCell in FR2-2)	5865

A.7.6.2.12.1	Test Purpose and Environment	5865

A.7.6.2.12.2	Test Requirements	5867

A.7.6.2.13	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is used (PCell in FR2-2)	5868

A.7.6.2.13.1	Test Purpose and Environment	5868

A.7.6.2.13.2	Test Requirements	5870

A.7.6.2.14	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is not used (PCell in FR2-2)	5871

A.7.6.2.14.1	Test Purpose and Environment	5871

A.7.6.2.14.2	Test Requirements	5873

A.7.6.2.15	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is used (PCell in FR2-2)	5874

A.7.6.2.15.1	Test Purpose and Environment	5874

A.7.6.2.15.2	Test Requirements	5876

A.7.6.2.16	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is not used (PCell in FR1)	5877

A.7.6.2.16.1	Test Purpose and Environment	5877

A.7.6.2.16.2	Test Requirements	5881

A.7.6.2.17	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is used (PCell in FR1)	5881

A.7.6.2.17.1	Test Purpose and Environment	5881

A.7.6.2.17.2	Test Requirements	5885

A.7.6.2.18	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is not used (PCell in FR1)	5886

A.7.6.2.18.1	Test Purpose and Environment	5886

A.7.6.2.18.2	Test Requirements	5889

A.7.6.2.19	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is used (PCell in FR1)	5890

A.7.6.2.19.1	Test Purpose and Environment	5890

A.7.6.2.19.2	Test Requirements	5894

A.7.6.2.20	SA event triggered reporting tests for FR2 with measurement gap with priority and two periodic MUSIM gaps configured	5895

A.7.6.2.20.1	Test Purpose and Environment	5895

A.7.6.2.20.2	Test Requirements	5897

A.7.6.2.21	SA event triggered reporting tests for FR2 with measurement gap without priority and periodic MUSIM gap configured	5897

A.7.6.2.21.1	Test Purpose and Environment	5897

A.7.6.2.21.2	Test Requirements	5899

A.7.6.2.22	SA event triggered reporting tests with SSB time index detection when DRX is not used (PCell in FR2) for FR2 power class 6 UE configured with highSpeedMeasFlagFR2-r17	5900

A.7.6.2.22.1	Test Purpose and Environment	5900

A.7.6.2.22.2	Test Requirements	5902

A.7.6.2.23	SA event triggered reporting tests without SSB time index detection when DRX is not used (PCell in FR2) for FR2 power class 6 UE configured with highSpeedMeasFlagFR2-r17	5902

A.7.6.2.23.1	Test Purpose and Environment	5902

A.7.6.2.23.2	Test Requirements	5904

A.7.6.3	L1-RSRP measurement for beam reporting	5904

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	5904

A.7.6.3.1.1	Test Purpose and Environment	5904

A.7.6.3.1.2	Test parameters	5904

A.7.6.3.1.3	Test Requirements	5906

A.7.6.3.2	SSB based L1-RSRP measurement when DRX is used	5906

A.7.6.3.2.1	Test Purpose and Environment	5906

A.7.6.3.2.2	Test parameters	5906

A.7.6.3.2.3	Test Requirements	5907

A.7.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	5908

A.7.6.3.3.1	Test Purpose and Environment	5908

A.7.6.3.3.2	Test parameters	5908

A.7.6.3.3.3	Test Requirements	5909

A.7.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	5909

A.7.6.3.4.1	Test Purpose and Environment	5909

A.7.6.3.4.2	Test parameters	5910

A.7.6.3.3.3	Test Requirements	5911

A.7.6.3.5	SSB based L1-RSRP measurement when DRX is used for power class 6 UE configured with highSpeedMeasFlagFR2-r17	5911

A.7.6.3.5.1	Test Purpose and Environment	5911

A.7.6.3.5.2	Test parameters	5912

A.7.6.3.5.3	Test Requirements	5913

A.7.6.3.6	Inter-cell SSB based L1-RSRP measurements on FR2 SCell when DRX is not used	5913

A.7.6.3.6.1	Test Purpose and Environment	5913

A.7.6.3.6.2	Test parameters	5914

A.7.6.3.6.3	Test Requirements	5916

A.7.6.3.7	SSB based L1-RSRP measurement for FR2-2 when DRX is used	5916

A.7.6.3.7.1	Test Purpose and Environment	5916

A.7.6.3.7.2	Test parameters	5917

A.7.6.3.7.3	Test Requirements	5918

A.7.6.3.8	CSI-RS based L1-RSRP measurement when DRX is not used and when CD-SSB is outside active BWP	5918

A.7.6.3.8.1	Test Purpose and Environment	5918

A.7.6.3.9	SSB based L1-RSRP measurement when DRX is not used when CD-SSB is outside active BWP	5919

A.7.6.3.9.1	Test Purpose and Environment	5919

A.7.6.3.9.2	Test Requirements	5919

A.7.6.3.10	SSB based L1-RSRP measurement for UE supporting NCD-SSB based L1 measurement outside active BWP when DRX is not used	5919

A.7.6.3.10.1	Test Purpose and Environment	5919

A.7.6.3.10.2	Test parameters	5919

A.7.6.3.10.3	Test Requirements	5921

A.7.6.3.11	SSB based L1-RSRP measurement when DRX is used for power class 6 UE supporting simultaneousReceptionTwoQCL-r18	5921

A.7.6.3.11.1	Test Purpose and Environment	5921

A.7.6.3.11.2	Test parameters	5921

A.7.6.3.11.3	Test Requirements	5923

A.7.6.4	CLI measurements	5923

A.7.6.4.1	SRS-RSRP measurement with non-DRX	5923

A.7.6.4.1.1	Test Purpose and Environment	5923

A.7.6.4.1.2	Test Parameters	5923

A.7.6.4.1.3	Test Requirements	5925

A.7.6.4.2	CLI-RSSI measurement with non-DRX	5925

A.7.6.4.2.1	Test Purpose and Environment	5925

A.7.6.4.2.2	Test Parameters	5926

A.7.6.4.2.3	Test Requirements	5927

A.7.6.5.1	SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2)	5927

A.7.6.5.1.1	Test Purpose and Environment	5927

A.7.6.5.1.2	Test Requirements	5930

A.7.6.6	L1-SINR measurement for beam reporting	5930

A.7.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	5932

A.7.6.6.2.1	Test Purpose and Environment	5932

A.7.6.6.2.2	Test parameters	5932

A.7.6.6.2.3	Test Requirements	5933

A.7.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	5934

A.7.6.6.3.1	Test Purpose and Environment	5934

A.7.6.6.3.2	Test parameters	5934

A.7.6.6.3.3	Test Requirements	5935

A.7.6.7	CSI-RS based intra-frequency Measurements	5936

A.7.6.7.1	SA event triggered reporting test without gap under DRX for CSI-RS based intra-frequency measurement	5936

A.7.6.7.1.1	Test purpose and Environment	5936

A.7.6.7.1.2	Test Requirements	5937

A.7.6.8	CSI-RS based inter-frequency Measurements	5938

A.7.6.8.1	SA event triggered reporting tests for FR2 CSI-RS based measurement when non-DRX is used (PCell in FR2)	5938

A.7.6.8.1.1	Test Purpose and Environment	5938

A.7.6.8.1.2	Test Requirements	5940

A.7.6.9	RSTD measurements	5940

A.7.6.9.1	 NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA	5940

A.7.6.9.1.1	Test Purpose and Environment	5940

A.7.6.9.1.2	Test Requirements	5944

A.7.6.9.2	 NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR2 SA	5944

A.7.6.9.2.1	Test Purpose and Environment	5944

A.7.6.9.2.2	Test Requirements	5947

A.7.6.9.3	NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA	5948

A.7.6.9.3.1	Test Purpose and Environment	5948

A.7.6.9.3.2	Test Requirements	5950

A.7.6.9.4	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA without measurement gap	5951

A.7.6.9.4.1	Test Purpose and Environment	5951

A.7.6.9.4.2	Test Requirements	5953

A.7.6.9.5	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_CONNECTED state with Rx TEG	5954

A.7.6.9.5.1	Test Purpose and Environment	5954

A.7.6.9.5.2	Test Requirements	5957

A.7.6.9.6	NR RSTD measurement reporting delay test case for PRS aggregation in FR2 SA in RRC_CONNECTED mode	5957

A.7.6.9.6.1	Test Purpose and Environment	5957

A.7.6.9.6.2	Test Requirements	5965

A.7.6.10 PRS-RSRP measurements	5965

A.7.6.10.1 PRS-RSRP reporting delay test case for single positioning frequency layer	5965

A.7.6.10.1.1	Test Purpose and Environment	5965

A.7.6.10.1.2	Test Requirements	5967

A.7.6.10.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	5968

A.7.6.10.2.1	Test Purpose and Environment	5968

A.7.6.10.2.2	Test Requirements	5970

A.7.6.10.3	PRS-RSRP reporting delay test case for reduced number of samples	5970

A.7.6.10.3.1	Test Purpose and Environment	5970

A.7.6.10.3.2	Test Requirements	5972

A.7.6.10.4	PRS-RSRP reporting delay test case for single positioning frequency layer outside MG	5973

A.7.6.10.4.1	Test Purpose and Environment	5973

A.7.6.10.4.2	Test Requirements	5975

A.7.6.11	UE Rx-Tx time difference measurements	5975

A.7.6.11.1	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	5975

A.7.6.11.1.1	Test purpose and environment	5975

A.7.6.11.1.2	Test requirements	5977

A.7.6.11.2	UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR2 SA	5977

A.7.6.11.2.1	Test purpose and environment	5977

A.7.6.11.2.2	Test requirements	5979

A.7.6.11.3	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA with reduced sample number	5980

A.7.6.11.3.1	Test purpose and environment	5980

A.7.6.11.3.2	Test requirements	5981

A.7.6.11.4	UE Rx-Tx time difference measurements without gaps in FR2 SA	5982

A.7.6.11.4.1	Test purpose and environment	5982

A.7.6.11.4.2	Test requirements	5983

A.7.6.11.5	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA with RxTx TEG	5984

A.7.6.11.5.1	Test purpose and environment	5984

A.7.6.11.5.2	Test requirements	5985

A.7.6.11.6	UE Rx-Tx time difference measurements with PRS bandwidth aggregation in FR2 SA	5986

A.7.6.11.6.1	Test purpose and environment	5986

A.7.6.11.6.2	Test requirements	5989

A.7.6.12	PRS-RSRPP measurements	5989

A.7.6.12.1 PRS-RSRPP reporting delay test case for single positioning frequency layer in FR2 in RRC_CONNECTED state	5989

A.7.6.12.1.1	Test Purpose and Environment	5989

A.7.6.12.1.2	Test Requirements	5992

A.7.6.12.2	PRS-RSRPP reporting delay test case for reduced number of samples for single positioning frequency layer in FR2 in RRC_CONNECTED state	5992

A.7.6.12.2.1	Test Purpose and Environment	5992

A.7.6.12.2.2	Test Requirements	5994

A.7.6.12.3	PRS-RSRPP reporting delay test case for gapless measurement in FR2	5994

A.7.6.12.3.1	Test Purpose and Environment	5994

A.7.6.12.3.2	Test Requirements	5997

A.7.6.13	UE Rx-Tx time difference measurements for PDC	5997

A.7.6.13.1	UE Rx-Tx time difference measurement for propagation delay compensation using PRS in FR2	5997

A.7.6.13.1.1	Test purpose and environment	5997

A.7.6.13.1.2	Test requirements	5998

A.7.6.13.2	UE Rx-Tx time difference measurement for propagation delay compensation using TRS in FR2	5999

A.7.6.13.2.1	Test purpose and environment	5999

A.7.6.13.2.2	Test requirements	6000

A.7.6.14	SA event triggered reporting tests with Pre-MG	6000

A.7.6.14.1	Intra-frequency measurement test with SA event triggered reporting tests: with autonomous activation/deactivation of Pre-MG in FR2	6000

A.7.6.14.1.1	Test purpose and Environment	6000

A.7.6.14.1.2	Test parameters	6001

A.7.6.14.1.3	Test Requirements	6002

A.7.6.14.2	Intra-frequency measurement test with SA event triggered reporting tests: with network-controlled activation/deactivation of Pre-MG in FR2	6003

A.7.6.14.2.1	Test purpose and Environment	6003

A.7.6.14.2.2	Test parameters	6003

A.7.6.14.2.3	Test Requirements	6005

A.7.6.15	SA event triggered reporting tests with concurrent gaps	6005

A.7.6.15.1	SA event triggered reporting tests For FR2 with fully non-overlapping concurrent MGs for SSB-based inter-frequency measurements	6005

A.7.6.15.1.1	Test Purpose and Environment	6005

A.7.6.15.1.2	Test Requirements	6007

A.7.6.15.2	SA event triggered reporting tests For FR2 with concurrent measurement gaps without SSB time index detection when DRX is not used (PCell in FR2)	6008

A.7.6.15.2.1	Test Purpose and Environment	6008

A.7.6.15.2.2	Test Requirements	6010

A.7.6.15.3	SA event triggered reporting tests for FR2 concurrent gap with partially partial overlapping scenario for SSB-based measurements and PRS-based measurement	6010

A.7.6.15.3.1	Test Purpose and Environment	6010

A.7.6.15.3.2	Test Requirements	6013

A.7.6.16	SA event triggered reporting tests with NCSG	6013

A.7.6.16.1	SA event triggered reporting test with per-UE NCSG under non-DRX	6013

A.7.6.16.1.1	Test purpose and Environment	6013

A.7.6.16.1.2	Test Requirements	6016

A.7.6.16.2	SA event triggered reporting tests on inter-frequency measurement with NCSG for FR2 when DRX is not used (PCell in FR2)	6016

A.7.6.16.2.1	Test Purpose and Environment	6016

A.7.6.16.2.2	Test Requirements	6018

A.7.6.16.3	Event triggered reporting test on deactivated SCell measurement via NCSG in FR2 in non-DRX	6019

A.7.6.16.3.1	Test Purpose and Environment	6019

A.7.6.16.3.2	Test Requirements	6021

A.7.6.17	SA event triggered reporting tests for concurrent measurement gaps with Pre-MG in FR2	6021

A.7.6.17.1	SA event triggered reporting test for FR2 with one pre-configured gap and one measurement gap	6021

A.7.6.17.1.1	Test Purpose and Environment	6021

A.7.6.17.1.2	Test Requirements	6023

A.7.6.17.2	Inter-frequency measurement test with SA event triggered reporting tests: with autonomous activation/deactivation of Pre-MGs in FR2	6024

A.7.6.17.2.1	Test purpose and Environment	6024

A.7.6.17.2.2	Test parameters	6024

A.7.6.17.2.3	Test Requirements	6026

A.7.6.18	SA event triggered reporting tests with concurrent gaps and NCSG	6027

A.7.6.18.1	SA event triggered reporting tests For FR2 with concurrent measurement gaps and NCSG without SSB time index detection when DRX is not used (PCell in FR2)	6027

A.7.6.18.1.1	Test Purpose and Environment	6027

A.7.6.18.1.2	Test Requirements	6029

A.7.6.19	SA event triggered reporting tests with NeedForGap in FR2	6030

A.7.6.19.1	SA event triggered reporting test for UE indicating NeedforInterruptionInfoNR under non-DRX and no interruption outside configured measurement gaps	6030

A.7.6.19.1.1	Test purpose and Environment	6030

A.7.6.19.1.2	Test Requirements	6032

A.7.6.19.2	SA event triggered reporting test without gap under non-DRX	6032

A.7.6.19.2.1	Test purpose and Environment	6032

A.7.6.19.2.2	Test Requirements	6034

A.7.6.19.3	SA event triggered reporting test without gap without interruption under non-DRX	6035

A.7.6.19.3.1	Test Purpose and Environment	6035

A.7.6.19.3.2	Test Requirements	6037

A.7.6.20	LTM Intra-frequency L1-RSRP measurement	6037

A.7.6.20.1	Intra-frequency SSB based L1-RSRP measurement in FR2	6037

A.7.6.20.1.1	Test Purpose and Environment	6037

A.7.6.20.1.2	Test parameters	6038

A.7.6.20.1.3	Test Requirements	6040

A.7.6.21	LTM Inter-frequency L1-RSRP measurement with measurement gap	6040

A.7.6.21.1	Inter-frequency SSB-based L1-RSRP measurement with measurement gap for LTM in FR2	6040

A.7.6.21.1.1	Test Purpose and Environment	6040

A.7.6.21.1.2	Test parameters	6040

A.7.6.21.1.3	Test Requirements	6042

A.7.6.22	LTM Inter-frequency L1-RSRP measurement without measurement gap	6042

A.7.6.22.1	Inter-frequency SSB based L1-RSRP measurement without measurement gap in FR2	6042

A.7.6.22.1.1	Test Purpose and Environment	6042

A.7.6.22.1.2	Test parameters	6043

A.7.6.22.1.3	Test Requirements	6045

A.7.6.23	Idle Mode CA/DC Measurements	6045

A.7.6.23.1	Test case for Idle mode fast CA/DC eEMR measurement for FR2 without valid reporting	6045

A.7.6.23.1.1	Test Purpose and Environment	6045

A.7.6.23.1.2	Test Requirements	6048

A.7.6.23.2	Test case for Idle mode fast CA/DC cell reselection measurement for FR2 without valid reporting	6048

A.7.6.23.2.1	Test Purpose and Environment	6049

A.7.6.23.2.2	Test Requirements	6052

A.7.6.23.3 Test case for Idle mode fast CA/DC cell reselection measurement for FR2 with valid reporting	6052

A.7.6.23.3.1	Test Purpose and Environment	6052

A.7.6.23.3.2	Test Requirements	6055

A.7.6.24	RSCPD measurements	6056

A.7.6.24.1	NR RSCPD with RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_CONNECTED state	6056

A.7.6.24.1.1	Test Purpose and Environment	6056

A.7.6.24.1.2	Test Requirements	6063

A.7.6.25	RSCP measurements	6063

A.7.6.25.1	DL RSCP with UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	6063

A.7.6.25.1.1	Test purpose and environment	6063

A.7.6.25.1.2	Test requirements	6067

A.7.7	Measurement Performance requirements	6067

A.7.7.1	SS-RSRP	6067

A.7.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	6067

A.7.7.1.1.1	Test Purpose and Environment	6067

A.7.7.1.1.2	Test parameters	6067

A.7.7.1.1.3	Test Requirements	6069

A.7.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	6070

A.7.7.1.2.1	Test Purpose and Environment	6070

A.7.7.1.2.2	Test parameters	6070

A.7.7.1.2.3	Test Requirements	6072

A.7.7.1.3	SA inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	6073

A.7.7.1.3.1	Test Purpose and Environment	6073

A.7.7.1.3.2	Test parameters	6073

A.7.7.1.3.3	Test Requirements	6075

A.7.7.2	SS-RSRQ	6076

A.7.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	6076

A.7.7.2.1.1	Test Purpose and Environment	6076

A.7.7.2.1.2	Test Parameters	6076

A.7.7.2.1.3	Test Requirements	6077

A.7.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	6078

A.7.7.2.2.1	Test Purpose and Environment	6078

A.7.7.2.2.2	Test Parameters	6078

A.7.7.2.2.3	Test Requirements	6079

A.7.7.3	SS-SINR	6079

A.7.7.3.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	6079

A.7.7.3.1.1	Test Purpose and Environment	6079

A.7.7.3.1.2	Test Parameters	6079

A.7.7.3.1.3	Test Requirements	6081

A.7.7.3.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	6081

A.7.7.3.2.1	Test Purpose and Environment	6081

A.7.7.3.2.2	Test Parameters	6081

A.7.7.3.2.3	Test Requirements	6082

A.7.7.4	L1-RSRP measurement for beam reporting	6083

A.7.7.4.1	SSB based L1-RSRP measurement	6083

A.7.7.4.1.1	Test Purpose and Environment	6083

A.7.7.4.1.2	Test parameters	6083

A.7.7.4.1.3	Test Requirements	6084

A.7.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	6085

A.7.7.4.2.1	Test Purpose and Environment	6085

A.7.7.4.2.2	Test parameters	6085

A.7.7.4.2.3	Test Requirements	6086

A.7.7.5	CLI measurements	6087

A.7.7.5.1	SA SRS-RSRP measurement accuracy with FR2 serving cell	6087

A.7.7.5.1.1	Test Purpose and Environment	6087

A.7.7.5.1.2	Test parameters	6087

A.7.7.5.1.3	Test Requirements	6089

A.7.7.5.2	SA CLI-RSSI measurement accuracy with FR2 serving cell	6089

A.7.7.5.2.1	Test Purpose and Environment	6089

A.7.7.5.2.2	Test parameters	6089

A.7.7.5.2.3	Test Requirements	6091

A.7.7.6	L1-SINR measurement for beam reporting	6091

A.7.7.6.1.1	Test Purpose and Environment	6091

A.7.7.6.1.2	Test parameters	6091

A.7.7.6.1.3	Test Requirements	6093

A.7.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	6093

A.7.7.6.2.1	Test Purpose and Environment	6093

A.7.7.6.2.2	Test parameters	6093

A.7.7.6.2.3	Test Requirements	6095

A.7.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	6095

A.7.7.6.3.1	Test Purpose and Environment	6095

A.7.7.6.3.2	Test parameters	6095

A.7.7.6.3.3	Test Requirements	6097

A.7.7.7	CSI-RSRP	6097

A.7.7.7.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	6097

A.7.7.7.1.1	Test Purpose and Environment	6097

A.7.7.7.1.2	Test parameters	6097

A.7.7.7.1.3	Test Requirements	6099

A.7.7.7.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	6100

A.7.7.7.2.1	Test Purpose and Environment	6100

A.7.7.7.2.2	Test parameters	6100

A.7.7.7.2.3	Test Requirements	6102

A.7.7.8	CSI-RSRQ	6103

A.7.7.8.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	6103

A.7.7.8.1.1	Test Purpose and Environment	6103

A.7.7.8.1.2	Test Parameters	6103

A.7.7.8.1.3	Test Requirements	6104

A.7.7.8.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	6104

A.7.7.8.2.1	Test Purpose and Environment	6104

A.7.7.8.2.2	Test Parameters	6104

A.7.7.8.2.3	Test Requirements	6106

A.7.7.9	CSI-SINR	6106

A.7.7.9.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	6106

A.7.7.9.1.1	Test Purpose and Environment	6106

A.7.7.9.1.2	Test Parameters	6106

A.7.7.9.1.3	Test Requirements	6108

A.7.7.9.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	6108

A.7.7.9.2.1	Test Purpose and Environment	6108

A.7.7.9.2.2	Test Parameters	6108

A.7.7.9.2.3	Test Requirements	6110

A.7.7.10	RSTD measurements	6110

A.7.7.10.1	RSTD measurement accuracy test case for single positioning frequency layer	6110

A.7.7.10.1.1	Test purpose and Environment	6110

A.7.7.10.1.2	Test Requirements	6111

A.7.7.10.2	RSTD measurement accuracy test case for dual positioning frequency layer	6111

A.7.7.10.2.1	Test purpose and Environment	6112

A.7.7.10.2.2	Test Requirements	6113

A.7.7.10.3	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR2 in RRC_CONNECTED state	6113

A.7.7.10.3.1	Test purpose and Environment	6113

A.7.7.10.3.2	Test Requirements	6115

A.7.7.10.4	RSTD measurement accuracy test case with Rx TEG	6115

A.7.7.10.4.1	Test purpose and Environment	6115

A.7.7.10.4.2	Test Requirements	6117

A.7.7.10.5	NR RSTD measurement accuracy test case for PRS aggregation in FR2 SA in RRC_CONNECTED mode	6117

A.7.7.10.5.1	Test purpose and Environment	6117

A.7.7.10.5.2	Test Requirements	6119

A.7.7.11	PRS-RSRP measurements	6119

A.7.7.11.1	SA measurement accuracy with PRS in FR2	6119

A.7.7.11.1.1	Test Purpose and Environment	6119

A.7.7.11.1.2	Test parameters	6119

A.7.7.11.1.3	Test Requirements	6121

A.7.7.11.2	SA measurement accuracy with PRS in FR2 with reduced sample number	6121

A.7.7.11.2.1	Test Purpose and Environment	6121

A.7.7.11.2.2	Test parameters	6121

A.7.7.11.2.3	Test Requirements	6123

A.7.7.12	UE Rx-Tx time difference measurements	6123

A.7.7.12.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR2 SA	6123

A.7.7.12.1.1	Test purpose and environment	6123

A.7.7.12.1.2	Test parameters	6124

A.7.7.12.1.3	Test requirements	6125

A.7.7.12.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR2 SA	6126

A.7.7.12.2.1	Test purpose and environment	6126

A.7.7.12.2.2	Test parameters	6126

A.7.7.12.2.3	Test requirements	6127

A.7.7.12.3	UE Rx-Tx time difference measurement accuracy with RxTx TEG	6127

A.7.7.12.3.1	Test purpose and environment	6127

A.7.7.12.3.2	Test parameters	6128

A.7.7.12.3.3	Test requirements	6129

A.7.7.12.4	UE Rx-Tx time difference measurement accuracy with PRS bandwidth aggregation in FR2 SA	6130

A.7.7.12.4.1	Test purpose and environment	6130

A.7.7.12.4.2	Test requirements	6134

A.7.7.13	PRS-RSRPP measurements	6134

A.7.7.13.1	SA measurement accuracy with PRS in FR2	6134

A.7.7.13.1.1	Test Purpose and Environment	6134

A.7.7.13.1.2	Test parameters	6134

A.7.7.13.1.3	Test Requirements	6136

A.7.7.13.2	SA measurement accuracy with reduced PRS samples in FR2	6136

A.7.7.13.2.1	Test Purpose and Environment	6136

A.7.7.13.2.2	Test parameters	6136

A.7.7.13.2.3	Test Requirements	6138

A.7.7.14	L1-RSRP measurement for group-based beam reporting	6138

A.7.7.14.1	SSB based L1-RSRP measurement	6138

A.7.7.14.1.1	Test Purpose and Environment	6138

A.7.7.14.1.2	Test parameters	6138

A.7.7.14.1.3	Test Requirements	6140

A.7.7.14.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	6140

A.7.7.14.2.1	Test Purpose and Environment	6140

A.7.7.14.2.2	Test parameters	6140

A.7.7.14.2.3	Test Requirements	6142

A.7.7.15	LTM L1-RSRP measurement	6142

A.7.7.15.1	SSB based inter-frequency L1-RSRP measurement	6142

A.7.7.15.1.1	Test Purpose and Environment	6142

A.7.7.15.1.2	Test parameters	6143

A.7.7.15.1.3	Test Requirements	6144

A.7.7.16	RSCPD Measurements	6145

A.7.7.16.1	RSCPD with RSTD measurement accuracy in FR2 SA in RRC_CONNECTED	6145

A.7.7.16.1.1	Test purpose and environment	6145

A.7.7.16.1.2	Test parameters	6145

A.7.7.16.1.3	Test requirements	6147

A.7.7.17	RSCP with UE Rx-Tx time difference measurements	6147

A.7.7.17.1	RSCP with UE Rx-Tx time difference measurement accuracy in FR2 SA	6147

A.7.7.17.1.1	Test purpose and environment	6147

A.7.7.17.1.2	Test parameters	6148

A.7.7.17.1.3	Test requirements	6151

A.7.8	Measurement procedure in RRC_INACTIVE	6151

A.7.8.1	RSTD measurements	6151

A.7.8.1.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_INACTIVE state	6151

A.7.8.1.1.1	Test Purpose and Environment	6151

A.7.8.1.1.2	Test Requirements	6154

A.7.8.1.2	NR RSTD measurement reporting delay test case with reduced number of samples in RRC_INACTIVE, FR1 SA	6154

A.7.8.1.2.1	Test Purpose and Environment	6154

A.7.8.1.2.2	Test Requirements	6157

A.7.8.1.3	NR RSTD measurement reporting delay test case for PRS aggregation in FR2 SA in RRC_INACTIVE state	6157

A.7.8.1.3.1	Test purpose and environment	6157

A.7.8.1.3.2	Test requirements	6161

A.7.8.1.4	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_INACTIVE state with eDRX > 10.24s	6161

A.7.8.1.4.1	Test purpose and environment	6161

A.7.8.1.4.2	Test requirements	6164

A.7.8.2	PRS-RSRP measurements	6164

A.7.8.2.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_INACTIVE	6164

A.7.8.2.1.1	Test Purpose and Environment	6164

A.7.8.2.1.2	Test Requirements	6166

A.7.8.2.2	PRS-RSRP reporting delay test case with reduced number of samples in RRC_INACTIVE	6167

A.7.8.2.2.1	Test purpose and Environment	6167

A.7.8.2.2.2	Test Requirements	6169

A.7.8.2.3	PRS-RSRP reporting delay in RRC_INACTIVE with eDRX	6169

A.7.8.2.3.1	Test Purpose and Environment	6169

A.7.8.2.3.2	Test Requirements	6173

A.7.8.3	UE Rx-Tx time difference measurements	6173

A.7.8.3.1	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	6173

A.7.8.3.1.1	Test purpose and environment	6173

A.7.8.3.1.2	Test requirements	6175

A.7.8.3.2	UE Rx-Tx time difference measurement with reduced number of samples in RRC_INACTIVE, FR2 SA	6175

A.7.8.3.2.1	Test purpose and environment	6175

A.7.8.3.2.2	Test requirements	6177

A.7.8.3.3	UE Rx-Tx time difference measurements with PRS bandwidth aggregation in FR2 SA	6177

A.7.8.3.3.1	Test purpose and environment	6177

A.7.8.3.3.2	Test requirements	6180

A.7.8.3.4	UE Rx-Tx time difference measurements for single positioning frequency layer with eDRX > 10.24s in FR2 SA	6180

A.7.8.3.4.1	Test purpose and environment	6180

A.7.8.3.4.2	Test requirements	6183

A.7.8.4	PRS-RSRPP measurements	6183

A.7.8.4.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR2 in RRC_INACTIVE state	6183

A.7.8.4.1.1	Test Purpose and Environment	6183

A.7.8.4.1.2	Test Requirements	6186

A.7.8.4.2	PRS-RSRPP reporting delay test with reduced number of samples for single positioning frequency layer in FR2 in RRC_INACTIVE state	6186

A.7.8.4.2.1	Test Purpose and Environment	6186

A.7.8.4.2.2	Test Requirements	6188

A.7.8.4.3	PRS-RSPP reporting delay in RRC_INACTIVE state with eDRX > 10.24s in FR2	6189

A.7.8.4.3.1	Test purpose and environment	6189

A.7.8.4.3.2	Test requirements	6192

A.7.8.5	RSCPD Measurements	6192

A.7.8.5.1	DL RSCPD reported with RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_INACTIVE state	6192

A.7.8.5.1.1	Test Purpose and Environment	6192

A.7.8.5.1.2	Test Requirements	6193

A.7.8.6	RSCP Measurements	6193

A.7.8.6.1	DL RSCP with UE Rx-Tx time difference measurements in RRC_INACTIVE for single positioning frequency layer in FR2 SA	6193

A.7.8.6.1.1	Test purpose and environment	6193

A.7.8.6.1.2	Test requirements	6197

A.7.9	Measurement performance requirements in RRC_INACTIVE	6197

A.7.9.1	RSTD measurements	6197

A.7.9.1.1	RSTD measurement accuracy test case for single positioning frequency layer in FR2 in RRC_INACTIVE state	6197

A.7.9.1.1.1	Test purpose and Environment	6197

A.7.9.1.1.2	Test Requirements	6199

A.7.9.1.2	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR2 in RRC_INACTIVE state	6199

A.7.9.1.2.1	Test purpose and Environment	6199

A.7.9.1.2.2	Test Requirements	6201

A.7.9.2	PRS-RSRP measurements	6203

A.7.9.2.1	SA measurement accuracy with PRS in FR2 in RRC_INACTIVE	6203

A.7.9.2.1.1	Test Purpose and Environment	6203

A.7.9.2.1.2	Test parameters	6203

A.7.9.2.1.3	Test Requirements	6204

A.7.9.2.2	PRS-RSRP measurements with reduced number of sample in RRC_INACTIVE	6205

A.7.9.2.2.1	Test Purpose and Environment	6205

A.7.9.2.2.2	Test parameters	6205

A.7.9.2.2.3	Test Requirements	6206

A.7.9.3	UE Rx-Tx time difference measurements	6207

A.7.9.3.1	UE Rx-Tx time difference measurements in RRC_INACTIVE	6207

A.7.9.3.1.1	Test purpose and environment	6207

A.7.9.3.1.2	Test parameters	6207

A.7.9.3.1.3	Test requirements	6208

A.7.9.3.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR2 SA	6208

A.7.9.3.2.1	Test purpose and environment	6208

A.7.9.3.2.2	Test parameters	6209

A.7.9.3.2.3	Test requirements	6210

A.7.9.3.3	UE Rx-Tx time difference measurement accuracy with PRS bandwidth aggregation in FR2 SA in RRC_INACTIVE state	6210

A.7.9.3.3.1	Test purpose and environment	6210

A.7.9.3.3.2	Test requirements	6214

A.7.9.4	PRS-RSRPP measurements	6214

A.7.9.4.1	SA measurement accuracy in FR2 in RRC INACTIVE	6214

A.7.9.4.1.1	Test Purpose and Environment	6214

A.7.9.4.1.2	Test parameters	6214

A.7.9.4.1.3	Test Requirements	6216

A.7.9.4.2	SA measurement accuracy with reduced PRS samples in FR2 in RRC INACTIVE	6216

A.7.9.4.2.1	Test Purpose and Environment	6216

A.7.9.4.2.2	Test parameters	6216

A.7.9.4.2.3	Test Requirements	6218

A.7.9.5	RSCPD Measurements	6218

A.7.9.5.1	RSCPD with RSTD measurement accuracy in FR2 SA in RRC_INACTIVE	6218

A.7.9.5.1.1	Test purpose and environment	6218

A.7.9.5.1.2	Test parameters	6218

A.7.9.5.1.3	Test requirements	6220

A.7.9.6	RSCP Measurements	6220

A.7.9.6.1	RSCP with UE Rx-Tx time difference measurement accuracy in FR2 SA	6220

A.7.9.6.1.1	Test purpose and environment	6220

A.7.9.6.1.2	Test parameters	6221

A.7.9.6.1.3	Test requirements	6222

A.7.10	Measurement Procedure in RRC_IDLE	6222

A.7.10.1	RSTD Measurements	6222

A.7.10.1.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_IDLE state for non-RedCap UE	6222

A.7.10.1.1.1	Test purpose and environment	6222

A.7.10.1.1.2	Test requirements	6225

A.7.10.1.2	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_IDLE state with eDRX > 10.24s	6225

A.7.10.1.2.1	Test purpose and environment	6225

A.7.10.1.2.2	Test requirements	6228

A.7.10.1.3	NR RSTD measurement reporting delay test case for PRS aggregation in FR2 SA in RRC_IDLE state	6228

A.7.10.1.3.1	Test purpose and environment	6228

A.7.10.1.3.2	Test requirements	6229

A.7.10.2	PRS-RSRP Measurements	6229

A.7.10.2.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_IDLE state for non-RedCap UE in FR2	6229

A.7.10.2.1.1	Test Purpose and Environment	6229

A.7.10.2.1.2	Test Requirements	6233

A.7.10.2.2	PRS-RSRP reporting delay test case in RRC_IDLE state in FR2 when eDRX cycle > 10.24s	6233

A.7.10.2.2.1	Test Purpose and Environment	6233

A.7.10.2.2.2	Test Requirements	6233

A.7.10.3	RSCPD Measurements	6234

A.7.10.3.1	DL RSCPD reported with RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_IDLE state	6234

A.7.10.3.1.1	Test Purpose and Environment	6234

A.7.10.3.1.2	Test Requirements	6234

A.7.11	Measurement Performance Requirements in RRC_IDLE	6235

A.7.11.1	RSTD Measurements	6235

A.7.11.1.1	NR RSTD measurement accuracy test case for single positioning frequency layer in FR2 SA in RRC_IDLE state for non-RedCap UE	6235

A.7.11.1.1.1	Test purpose and environment	6235

A.7.11.1.1.2	Test requirements	6236

A.7.11.1.2	RSTD measurement accuracy test case for single positioning frequency layer in FR2 SA in RRC_IDLE state with eDRX > 10.24s	6237

A.7.11.1.2.1	Test purpose and environment	6237

A.7.11.1.2.2	Test requirements	6238

A.7.11.1.3	NR RSTD measurement accuracy test case for PRS aggregation in FR2 SA in RRC_IDLE state	6239

A.7.11.1.3.1	Test purpose and environment	6239

A.7.11.1.3.2	Test requirements	6239

A.7.11.2	PRS-RSRP measurements	6239

A.7.11.2.1	PRS-RSRP measurement accuracy test case for non-RedCap UE in FR2 in RRC_IDLE state	6239

A.7.11.2.1.1	Test Purpose and Environment	6239

A.7.11.2.1.2	Test parameters	6239

A.7.11.2.1.3	Test Requirements	6241

A.7.11.2.2	PRS-RSRP measurement accuracy test case in RRC_IDLE state in FR2 for case 2 when eDRX cycle > 10.24s	6241

A.7.11.2.2.1	Test purpose and Environment	6241

A.7.11.2.2.1	Test parameters	6242

A.7.11.2.2.2	Test Requirements	6242

A.7.11.3	RSCPD measurements	6242

A.7.11.3.1	RSCPD with RSTD measurement accuracy in FR2 SA in RRC_IDLE	6242

A.7.11.3.1.1	Test purpose and environment	6242

A.7.11.3.1.2	Test parameters	6242

A.7.11.3.1.3	Test requirements	6244

Annex B (normative):	Conditions for RRM requirements applicability for operating bands	6246

B.1	Conditions for NR RRC_IDLE state mobility	6246

B.1.1	Introduction	6246

B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection	6246

B.1.2A	Conditions for measurements on NR intra-frequency cells under CCA for cell re-selection	6248

B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection	6249

B.1.3A	Conditions for measurements on NR inter-frequency cells under CCA for cell re-selection	6249

B.1.4	Conditions for measurements on NR intra-frequency cells for cell re-selection for RedCap	6249

B.1.5	Conditions for measurements on NR inter-frequency cells for cell re-selection for RedCap	6252

B.1.6	Conditions for measurements on NR intra-frequency cells for cell re-selection for satellite access	6252

B.1.7	Conditions for measurements on NR inter-frequency cells for cell re-selection for satellite access	6252

B.2	Conditions for UE measurements procedures and performance requirements in RRC_CONNECTED state	6252

B.2.1	Introduction	6252

B.2.1.1	General	6252

B.2.1.2	Derivation of Minimum SSB_RP values for FR1	6253

B.2.1.3	Derivation of Minimum SSB_RP values for FR2	6253

B.2.1.3.1	Minimum SSB_RP values for Rx Beam Peak angle of arrival	6253

B.2.1.3.2	Minimum SSB_RP values for angle of arrival within Spherical coverage	6254

B.2.1.4	Gain to SS-RSRP and CSI-RSRP measurement point for FR1	6254

B.2.1.5	Gain to SS-RSRP and CSI-RSRP measurement point for FR2	6254

B.2.1.5.1	Gain to SS-RSRP and CSI-RSRP measurement point for Rx Beam Peak angle of arrival	6254

B.2.1.5.2	Gain to SS-RSRP measurement point for different frequency	6255

B.2.1.5.3	Alignment of Rough beam to Rx beam Peak	6255

B.2.1.6	Gain to PRS-RSRP measurement point for FR2	6256

B.2.1.6.1	Gain to PRS-RSRP measurement point for Rx Beam Peak angle of arrival	6256

B.2.1.7	Derivation of Minimum SSB_RP values for FR2-NTN for satellite access	6256

B.2.1.7.1	Minimum SSB_RP values for Rx Beam	6256

B.2.1.8	Gain to SS-RSRP for FR2-NTN for satellite access	6257

B.2.2	Conditions for NR intra-frequency measurements	6258

B.2.3	Conditions for NR inter-frequency measurements	6260

B.2.4	Conditions for NR L1-RSRP reporting	6262

B.2.4.1	Conditions for SSB based L1-RSRP reporting	6262

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting	6264

B.2.5	Conditions for RRC connection release with redirection to NR	6266

B.2.6	Void	6268

B.2.6.1	Void	6268

B.2.6.2	Void	6268

B.2.7	Conditions for SRS-RSRP measurements	6268

B.2.8	Conditions for NR L1-SINR reporting	6269

B.2.8.1	Conditions for L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	6269

B.2.8.2	Conditions for L1-SINR reporting with SSB based CMR and dedicated IMR configured	6271

B.2.8.2.1	L1-SINR reporting with SSB based CMR and dedicated ZP-IMR configured	6271

B.2.8.2.2	L1-SINR reporting with SSB based CMR and dedicated NZP-IMR configured	6273

B.2.8.3	Conditions for L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	6275

B.2.8.3.1	L1-SINR reporting with CSI-RS based CMR and dedicated ZP-IMR configured	6275

B.2.8.3.2	L1-SINR reporting with CSI-RS based CMR and dedicated NZP-IMR configured	6277

B.2.9	Conditions for NR intra-frequency measurements under CCA	6279

B.2.10	Conditions for NR inter-frequency measurements under CCA	6279

B.2.11	Conditions for NR L1-RSRP reporting under CCA	6279

B.2.11.1	Conditions for SSB based L1-RSRP reporting	6279

B.2.12	Conditions for NR CSI-RS based intra-frequency measurements	6280

B.2.13	Conditions for NR CSI-RS based inter-frequency measurements	6281

B.2.14	Conditions for NR PRS-based measurements	6282

B.2.15	Conditions for NR intra-frequency measurements for RedCap	6284

B.2.16	Conditions for NR inter-frequency measurements for RedCap	6285

B.2.17	Conditions for NR intra-frequency measurements for satellite access	6287

B.2.18	Conditions for NR inter-frequency measurements for satellite access	6287

B.2.19	Conditions for NR L1-RSRP reporting for satellite access	6287

B.2.19.1	Conditions for SSB based L1-RSRP reporting for satellite access	6288

B.2.19.2	Conditions for CSI-RS based L1-RSRP reporting for satellite access	6288

B.2.20	Conditions for RRC connection release with redirection to NR for satellite access	6288

B.3	RRM Requirements Exceptions	6288

B.3.1	Introduction	6288

B.3.2	Receiver sensitivity relaxation for CA	6289

B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1	6289

B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1	6289

B.3.2.2.1	Inter-band carrier aggregation	6289

B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA	6289

B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	6289

B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2	6290

B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2	6290

B.3.2.4.1	Intra-band contiguous carrier aggregation	6290

B.3.2.4.2	Intra-band non-contiguous carrier aggregation	6290

B.3.3	Receiver sensitivity relaxation for DC	6290

B.3.3.1	Receiver sensitivity relaxation for EN-DC	6290

B.3.3.2	Receiver sensitivity relaxation for NE-DC	6290

B.3.4	Receiver sensitivity relaxation for SUL	6290

B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1	6290

B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1	6290

B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL	6290

B.4	Conditions for V2X	6291

B.4.1	Test parameters for GNSS signals	6291

B.4.2	Conditions for PSBCH-RSRP Accuracy Requirements	6291

B.4.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	6291

B.4.4	Conditions for L1 SL-RSRP Accuracy Requirements	6292

B.4.5	Conditions for PSBCH-RSRP Accuracy Requirements under CCA	6292

B.4.6	Conditions for Selection/Reselection to Intra-frequency SyncRef UE under CCA	6292

B.4.7	Conditions for L1 SL-RSRP Accuracy Requirements under CCA	6293

B.4A	Conditions for NR Sidelink Positioning Measurement Procedures and Performance Requirements	6293

B.4A.1	Conditions for NR SL-PRS based measurements	6293

B.5	High level test procedure for SAN RRM tests	6293

Annex C (informative):	Change history	6295