| 3GPP TS 38.291 V19.2.0 (2025-12) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Ambient IoT Physical layer (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	8

4	Time and frequency domain structures	8

4.1	Chips	8

4.2	D2R	8

4.3	R2D	8

4.3.1	General	8

4.3.2	Time domain	9

4.3.3	Physical resources	9

4.3.3.1	Resource grid	9

4.3.3.2	Resource elements	9

4.3.3.3	Physical resource blocks	9

5	Mapping to physical channels	9

6	Physical channels and signals generation	10

6.1	D2R	10

6.1.1	Overview	10

6.1.2	PDRCH	11

6.1.2.1	CRC attachment	11

6.1.2.2	Block repetition	11

6.1.2.3	Channel coding or omission	12

6.1.3	D2R-amble insertion	12

6.1.4	Modulation for small frequency shift and mapping to chips	12

6.1.5	Backscattering	13

6.2	R2D	13

6.2.1	Overview	13

6.2.2	PRDCH	14

6.2.2.1	CRC attachment	14

6.2.2.2	Line encoding	14

6.2.3	Timing acquisition signal	14

6.2.3.1	Start indicator part	14

6.2.3.2	Clock acquisition part	14

6.2.4	Postamble	14

6.2.5	Mapping to chips and modulation	14

6.2.6	Mapping to OFDM symbols	15

6.2.7	Baseband signal generation	15

6.2.8	Modulation and upconversion	15

7	Physical layer procedures	15

7.1	D2R related procedures	15

7.1.1	Device procedure for D2R generation	15

7.1.2	Device procedure for transmission time determination	16

7.1.3	Device procedure for modulation scheme determination	17

7.2	R2D related procedures	17

7.2.1	Device procedure for R-TAS reception	17

7.2.2	Device procedure for PRDCH reception	17

7.2.3	Monitoring of R2D	18

7.2.4	End of R2D	18

8	Generic functions	18

8.1	CRC calculation	18

8.2	Channel coding	18

8.3	Sequence generation	19

8.4	Modulation mapping	19

8.4.1	OOK	19

8.4.1.1	OOK for small frequency shift	19

8.4.1.2	OOK for R2D	19

8.4.2	BPSK	19

8.5	Line encoding	19

Annex A (informative):	Change history	20

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

This document specifies the physical layer of Ambient IoT.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.211: "NR; Physical channels and modulation".

[3]	3GPP TS 38.391: "Ambient IoT Medium Access Control (MAC) protocol".

[4]	3GPP TS 38.194: "Ambient IoT Base Station (BS) and Carrier-Wave (CW) node radio transmission and reception".

[5]	3GPP TS 38.191: "Ambient IoT device radio transmission and reception".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Δf	Subcarrier spacing in R2D

Ibit	Interval for insertion of a D2R midamble

kRE	Index of a subcarrier relative to a reference in R2D

lamble	Length of a D2R amble sequence

lRE	OFDM symbol index relative to a reference in R2D

Mchip 	Number of chips to transmit

Mchipsymb	Number of chips per OFDM symbol for the R-TAS CAP, PRDCH, R2D postamble and padding

NCAP	Number of bits in the R-TAS CAP

NCP,lRE	Cyclic prefix length in R2D

Npad	Number of padding chips

NRBmin	Minimum number of physical resource blocks to be used in R2D

NRBR2D	Number of physical resource blocks used in R2D

NSFS	Number of potential small frequency shifts

NSIP	Number of bits in the R-TAS SIP

NscRB	Number of subcarriers per resource block in R2D

NTBSD2R	Size of a D2R transport block

NTBSR2D	Size of an R2D transport block

nPRB	Physical resource block number in R2D

Rblock	Block repetition number

RSFS	Small frequency shift factor

TbitD2R	Duration of a D2R bit for transmission

Tc	Basic time unit for R2D

TchipD2R	Duration of a D2R chip

TchipR2D	Duration of a chip for the R-TAS CAP, PRDCH, R2D postamble and padding

TD→Rmin	Minimum timing relationship from D2R to R2D

TR→D	Timing relationship from R2D to D2R

χ	Chip index relative to a reference

zkRE,lRE	Value of resource element kRE,lRE in R2D

zχ	Value of chip χ in D2R

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

A-IoT	Ambient IoT

CAP	Clock acquisition part of R-TAS

CRC	Cyclic redundancy check

D2R	Device to reader

DFT-s-OFDM	DFT spread OFDM

IoT	Internet of Things

PDRCH	Physical device-to-reader channel

PRDCH	Physical reader-to-device channel

R-TAS	R2D timing acquisition signal

R2D	Reader to device

SIP	Start indicator part of R-TAS

# 4	Time and frequency domain structures

## 4.1	Chips

From the device perspective, a transmission in D2R or a reception in R2D in the time domain consists of Mchipq consecutive chips each uniquely identified by the index χq=0, 1,…,Mchipq-1. χ refers to the chip position in the time domain relative to some reference point. The superscript q is set to D2R or R2D respectively, and may be omitted where there is no risk of confusion.

## 4.2	D2R

D2R transmissions are organised into consecutive chips each of duration TchipD2R.

D2R chip χ corresponds to the value zχ.

The constant τ=2×106/15000.

## 4.3	R2D

### 4.3.1	General

The size of various fields in the time domain for R2D is expressed in time units Tc=1/(480×103×4096) seconds. The constant κ=64.

### 4.3.2	Time domain

R2D transmissions are organised into OFDM symbols.

There are Mchipsymb consecutive chips each of duration TchipR2D=1MchipsymbΔf seconds during each OFDM symbol for the R-TAS CAP, PRDCH, the R2D postamble and padding if needed.

There are 4 consecutive chips each of duration 14Δf seconds during each OFDM symbol for the R-TAS SIP.

### 4.3.3	Physical resources

#### 4.3.3.1	Resource grid

The transmitted signal in each OFDM symbol is described by a resource grid of NRBR2DNscRB subcarriers. The frequency location of a subcarrier refers to the centre frequency of that subcarrier.

The subcarrier spacing Δf=15×103 Hz.

#### 4.3.3.2	Resource elements

Each element in the resource grid is called a resource element and is uniquely identified by the index pair (kRE,lRE), where kRE=0, 1, …, NRBR2DNscRB-1 is the index in the frequency domain and lRE=0 and up refers to the symbol position in the time domain relative to some reference point.

Resource element kRE,lRE corresponds to the value zkRE,lRE.

For in-band deployment, the starting position of OFDM symbol lRE=0 is aligned in time with the starting position of an NR OFDM symbol as specified in clause 5.3.1 of TS 38.211 [2].

#### 4.3.3.3	Physical resource blocks

A physical resource block is defined as NscRB=12 consecutive subcarriers in the frequency domain. Physical resource blocks are numbered from 0 to NRBR2D-1. The relation between the physical resource block nPRB and subcarrier kRE is:

nPRB=kRENscRB.

NRBR2D≥NRBmin , where NRBmin=1 for R-TAS SIP, and as shown in Table 4.3.3.3-1 for PRDCH, R-TAS CAP, R2D postamble and padding transmissions.

Table 4.3.3.3-1: Minimum number of PRBs for PRDCH, R-TAS CAP, R2D postamble and padding transmissions

| Number of chips per OFDM symbol Mchipsymb | NRBmin |
| --- | --- |
| 2 | 1 |
| 6 | 1 |
| 12 | 2 |
| 24 | 3 |

# 5	Mapping to physical channels

Table 5-1 specifies the mapping of the D2R and R2D transport channels to their corresponding physical channels.

Table 5-1

| TrCH | Physical Channel |
| --- | --- |
| D2R-TrCH | PDRCH |
| R2D-TrCH | PRDCH |

# 6	Physical channels and signals generation

Transport blocks from/to the MAC layer are encoded/decoded to offer transport and control services over the radio transmission link.

## 6.1	D2R

### 6.1.1	Overview

A D2R channel corresponds to a set of chips carrying information originating from higher layers. A D2R signal is used by the physical layer, but does not carry information originating from higher layers. The following D2R physical channel and signals are defined:

-	Physical device-to-reader channel, PDRCH

-	D2R-amble signals consisting of the D2R preamble and D2R midamble

D2R generation has the steps shown in the following figure.

D2R transport block

CRC attachment

Block repetition

a0,a1,…aA-1

b0,b1,…bB-1

c0,c1,…cBR-1

Channel coding

D2R-amble insertion

e0,e1,…eE-1

v0,v1,…vV-1

Modulation for small frequency shift

y0,y1,…yMchip-1

Mapping to chips χD2R

Figure 6.1.1-1: D2R steps

### 6.1.2	PDRCH

Data arrives to the generation unit in the form of a block of bits from higher layers, termed a D2R transport block.

#### 6.1.2.1	CRC attachment

Error detection is provided for the transport block through a CRC.

The entire transport block shall be used to calculate the CRC parity bits. Denote the bits in a transport block delivered to L1 by a0,a1,…, aA-1, and the parity bits by π0,π1,…, πL-1, where A is the payload size and L is the number of parity bits. The lowest order information bit a0 is mapped to the most significant bit of the transport block as defined in clause 6.1.1 of TS 38.391 [3].

The parity bits shall be computed and attached to the transport block according to clause 8.1, by setting L to 16 bits if A>24; and by setting L to 6 bits otherwise.

The bits after CRC attachment are denoted b0,b1,…, bB-1, where B=A+L.

#### 6.1.2.2	Block repetition

The bits input to the block repetition step are denoted b0,b1,…, bB-1, where B is the number of bits in the transport block including CRC parity bits.

Block repetition shall be performed so that the bits after repetition are denoted c0,c1,…, cBR-1 where BR=RblockB,  Rblock ≥1 is the block repetition number and:

ck+KB=bk
k=0, 1, …, B-1
K=0, 1, …, Rblock-1.

#### 6.1.2.3	Channel coding or omission

Only if Rcode indicates that no channel coding is used, the bits after block repetition are re-denoted e0,e1,…, eE-1, where E=BR, and the remainder of this clause shall be omitted.

Otherwise, one code block is delivered to the channel coding block. The bits in the code block are denoted c0,c1,…, cBR-1 where BR is the number of bits present after block repetition. The code block shall be encoded according to clause 8.2.

After encoding the bits are denoted d0i,d1i, …, dD-1i, i=0,1,2, where i is the coded stream index and D is the number of bits in each coded stream, i.e., D=BR.

The sequence of bits e0,e1,…, eE-1 for transmission, where E=3D, is generated by:

e3k+i=dki for k=0, 1, …,D-1 and i=0, 1, 2.

### 6.1.3	D2R-amble insertion

The bits of the D2R preamble signal, denoted p0,p1,…, plamble-1, shall be generated according to clause 8.3 by setting MPN=lamble.

The bits of the D2R midamble signal, denoted m0,m1, …, mlamble-1, shall be generated according to clause 8.3 by setting MPN=lamble.

An assembly of bits denoted v0,v1,…, vV-1, where

V=E+1+EIbitlambleif Iadd indicates insertion of an additional D2R midambleE+1+EIbitlambleotherwise,

is defined on the PDRCH bits ek, D2R preamble signal bits pk and D2R midamble signal bits mk as follows:

-	The bits p0,p1,…, plamble-1 of the D2R preamble are arranged into vk=pk for k=0, 1, …, lamble-1.

-	If E≥Ibit, the bits m0,m1, …, mlamble-1 of the D2R midamble signal are arranged into vk according to:

vk+Klamble+Ibit=mk
k=0, 1, …, lamble-1
K=1, 2, …, EIbit.

-	If Iadd indicates insertion of an additional D2R midamble, the bits m0, m1, …, mlamble-1 of the D2R midamble signal are arranged into vk+V-lamble=mk for k=0, 1, …, lamble-1.

-	The PDRCH bits ek for k=0, 1, …, E-1 are arranged into all bits of v which are not occupied by the preamble or a midamble.

### 6.1.4	Modulation for small frequency shift and mapping to chips

The assembly of bits v shall be modulated for small frequency-shift according to clause 8.4.1.1 or 8.4.2 using a small frequency shift factor RSFS, after which the set of modulated symbols is denoted y0,y1,…, yMchip-1. The modulated symbols shall be mapped in sequence starting with y0 to chips χ in increasing order.

### 6.1.5	Backscattering

The time-continuous signal st=zχ, where t=0 at the start of chip χ=0. The carrier-wave for backscattering at frequency f0 defined to start at time tCW=0 is denoted by wf0,tCW as specified in TS 38.194 [4].

The backscattered signal on the carrier wave is given by:

stwf0,tCW.

## 6.2	R2D

### 6.2.1	Overview

An R2D channel corresponds to a set of chips carrying information originating from higher layers. An R2D signal is used by the physical layer, but does not carry information originating from higher layers. The following R2D physical channel and signals are defined:

-	Physical reader-to-device channel, PRDCH

-	R2D timing acquisition signal, R-TAS, which consists of a start indicator part, SIP, and a clock acquisition part, CAP.

-	R2D postamble signal

R2D generation has the steps shown in the following figure.

R2D transport block

CRC attachment

a0,a1,…aA-1

b0,b1,…bB-1

Line encoding

c0,c1,…c2B-1

Mapping to OFDM symbols

R-TAS and postamble insertion

Padding (if needed)

Mapping to chips

Modulation

χR2D

Figure 6.2.1-1: R2D steps

### 6.2.2	PRDCH

Data arrives to the generation unit in the form of a block of bits from higher layers, termed an R2D transport block.

#### 6.2.2.1	CRC attachment

Error detection is provided for the transport block through a CRC.

The entire transport block is used to calculate the CRC parity bits. Denote the bits in a transport block delivered to L1 by a0,a1,…, aA-1, and the parity bits by π0,π1,…, πL-1, where A is the payload size and L is the number of parity bits. The lowest order information bit a0 is mapped to the most significant bit of the transport block as defined in clause 6.1.1 of TS 38.391 [3].

The parity bits are computed and attached to the transport block according to clause 8.1, by setting L to 16 bits if A>24; and by setting L to 6 bits otherwise.

The bits after CRC attachment are denoted b0,b1,…, bB-1, where B=A+L.

#### 6.2.2.2	Line encoding

The bits input to the line encoding step are denoted b0,b1,…, bB-1, where B is the number of bits in the transport block including CRC parity bits. The bits are line encoded according to clause 8.5.

After line encoding, the bits corresponding to the line codewords are denoted c0,c1,…, c2B-1.

### 6.2.3	Timing acquisition signal

#### 6.2.3.1	Start indicator part

The R-TAS SIP consists of NSIP=8 bits denoted S0,S1,S2, S3,S4,S5,S6,S7=1, 1, 0, 0, 1, 0, 0, 0.

#### 6.2.3.2	Clock acquisition part

The R-TAS CAP consists of NCAP=4 bits denoted A0,A1,A2,A3=1, 0, 1, 0.

### 6.2.4	Postamble

The R2D postamble signal consists of 4 bits denoted P0,P1,P2,P3=1, 1, 1, 1.

### 6.2.5	Mapping to chips and modulation

To chips χ=0 and up are mapped:

-	the bits of the R-TAS SIP in sequence starting with S0 followed by

-	the bits of the R-TAS CAP in sequence starting with A0 followed by

-	the bits of PRDCH in sequence starting with c0 followed by

-	the bits of the R2D postamble in sequence starting with P0,

except if Mchipsymb=24, chips χ=χ', χ'+1 satisfying χ'-NSIP modulo Mchipsymb=Mchipsymb-2 are skipped for the mapping of PRDCH and the R2D postamble, and are instead set to values of 1.

The content of each chip is modulated according to clause 8.4.1.2.

Following the last modulated symbol, the smallest integer Npad≥0 padding chips are inserted, if needed, until MchipR2D -NSIP modulo Mchipsymb=0. The padding chips are set to any values which do not result in another R-TAS SIP, and if Mchipsymb=24 values of 1 are mapped to the final two padding chips.

### 6.2.6	Mapping to OFDM symbols

Chips χ=0, 1,…,NSIP-1 are mapped in sequence to OFDM symbols lRE=χ/4.

Chips χ=NSIP, NSIP+1, …, MchipR2D-1  are mapped in sequence to OFDM symbols lRE= χ-NSIP/Mchipsymb+2 in increasing order.

### 6.2.7	Baseband signal generation

There is no specified relationship between chip χ mapped to OFDM symbol lRE and the value zkRE,lRE corresponding to resource element (kRE,lRE).

The time continuous signal slRE(t) for OFDM symbol lRE is defined by:

slREt=slRE(t)tstart,lRE≤t<tstart,lRE+Tsymb,lRE0otherwise
slREt=kRE=0k-1zkRE,lREej2πkRE-k2Δft-NCP,lRETc-tstart,lRE

Tsymb,lRE=Nu+NCP,lRETc

where NRBR2DNscRB≥k≥NRBminNscRB, t=0 at the start of OFDM symbol lRE=0 and

Nu=2048κ
NCP,lRE=160κlRE modulo 7=l0144κotherwise

where l0 is a value from {0,1,2,3,4,5,6}, constant for all OFDM symbols of the generation.

The starting position of OFDM symbol lRE is given by:

tstart,lRE=0lRE=0tstart,lRE-1+Tsymb,lRE-1otherwise.

### 6.2.8	Modulation and upconversion

Modulation and upconversion to the carrier frequency f0 of the complex-valued DFT-s-OFDM baseband signal for OFDM symbol lRE assumed to start at t=0 is given by:

ReslRE(t)∙ej2πf0t-tstart,lRE-NCP,lRETc .

# 7	Physical layer procedures

## 7.1	D2R related procedures

### 7.1.1	Device procedure for D2R generation

A device shall generate the D2R transmission using the following parameters provided by higher layers:

-	the duration in microseconds of each D2R bit, TbitD2R

-	the block repetition number, Rblock

-	the small frequency shift factor to be used, RSFS

-	the interval in bits for D2R midamble insertion, Ibit

-	sequence length indicator for D2R-ambles, Lamble

-	the additional D2R midamble insertion indicator, Iadd

-	the channel coding indicator, Rcode

-	the D2R transport block size in bytes, NTBSD2R

The device shall:

-	set TchipD2R=TbitD2R/2×RSFS

-	if Lamble indicates a short D2R amble sequence, set lamble=7; otherwise set lamble=31.

### 7.1.2	Device procedure for transmission time determination

A device shall upon receiving a PRDCH intended for the device in an R2D transmission ending in chip χendR2D=MchipR2D-1, perform a corresponding D2R transmission with chip χD2R=0 starting an amount of time TR→D after the end of chip χR2D= χendR2D according to the configuration received from higher layers.

If the D2R transmission is for an Access Random ID message (Msg1) or corresponds to a Random ID Response message (Msg2)

-	the device shall determine the D2R transmission starting time using the following parameters determined by higher layers:

-	the set of NSFS≥1 potential small frequency shift factors Ri,SFS', i=1, 2, …, NSFS

-	Tchip' is equal to the largest value among TbitD2R/(2×Ri,SFS')

otherwise

-	Tchip'=TchipD2R.

If the D2R transmission is for an Access Random ID message

-	if after chip χR2D= χendR2D  there are potential access occasion(s), as defined in TS 38.391 [3], for the transmission which are earlier in time than the access occasion selected for the transmission

-	the device shall set TR→D=1.25Toffset+MchipD2RTchipD2R

-	otherwise

-	the device shall set TR→D=Toffset

else if the D2R transmission corresponds to a R2D Random ID Response message or to a contention-free access procedure

-	the device shall set TR→D=Toffset+Δcode where Δcode has the value given in Table 7.1.2-1 if Rcode indicates that channel coding is used, and TR→D=Toffset if no channel coding is used

Table 7.1.2-1

| Tchip' [μs] | Δcode [μs] |
| --- | --- |
| τ | 0 |
| τ/2 |  |
| τ/4 |  |
| τ/8 |  |
| τ/16 |  |
| τ/32 | τ |
| τ/64 |  |
| τ/128 |  |
| τ/192 |  |

otherwise

-	the device shall set TR→D=Toffset+Δcode where Δcode has the value given in Table 7.1.2-2 if Rcode indicates that channel coding is used, and TR→D=Toffset if no channel coding is used

Table 7.1.2-2

| D2R block size [bytes] | Δcode [μs] |
| --- | --- |
| NTBSD2R≤32 | 2τ |
| 32<NTBSD2R≤64 | 4τ |
| 64<NTBSD2R≤125 | 8τ |

where Toffset has the value given in Table 7.1.2-3 and Mchipsymb is as determined for the corresponding PRDCH.

Table 7.1.2-3

| Tchip' [μs] | Toffset [μs] |
| --- | --- |
| τ | 10τ |
| τ/2 |  |
| τ/4 | 5τ |
| τ/8 |  |
| τ/16 |  |
| τ/32 | τ |
| τ/64 | τ, if Mchipsymb=2 τ/4, if Mchipsymb=6 or 12 or 24 |
| τ/128 |  |
| τ/192 |  |

### 7.1.3	Device procedure for modulation scheme determination

To determine the modulation scheme for the entire D2R transmission, the device shall:

-	if the PDRCH is for transmitting Msg1 or corresponds to a contention-free access procedure

-	determine according to its implementation to use either OOK modulation or BPSK modulation

-	otherwise

-	use the same modulation as determined for transmitting the preceding PDRCH.

## 7.2	R2D related procedures

### 7.2.1	Device procedure for R-TAS reception

A device shall, upon determining that a SIP of R-TAS has been received, determine the value of Mchipsymb according to which CAP is received from among those given in Table 4.3.3.3-1.

### 7.2.2	Device procedure for PRDCH reception

A device shall, upon receiving R-TAS, assume that a PRDCH transmission begins in chip χ=χstartPRDCH, receive the assumed PRDCH and attempt to decode the corresponding R2D transport block, where:

-	χstartPRDCH=NSIP+NCAP

-	the device can assume the R2D transport block size is NTBSR2D as specified in TS 38.391 [3], if the value is indicated by higher layers.

### 7.2.3	Monitoring of R2D

A device is not required to monitor R2D:

-	for the duration between the end of chip χR2D=χendR2D and the start of chip χD2R=0 as determined in Clause 7.1.2

-	when performing D2R transmission

-	for a duration TD→Rmin, as specified in TS 38.191 [5], after the end of chip χD2R=χendD2R when the device performs a D2R transmission ending in chip χendD2R=MchipD2R-1.

### 7.2.4	End of R2D

A device assumes there is a transition edge at the end of chip χR2D=χendR2D.

# 8	Generic functions

## 8.1	CRC calculation

Denote the input bits to the CRC computation by a0,a1,…, aA-1 and the parity bits by π0, π1, …, πL-1, where A  is the size of the input sequence and L is the number of parity bits. The parity bits are generated by one of the following cyclic generator polynomials:

-	gCRC16D=D16+D12+D5+1 for a CRC length L=16.

-	gCRC6D=D6+D5+1 for a CRC length L=6.

The encoding is performed in a systematic form, which means that in GF(2), the polynomial:

a0DA+L-1+a1DA+L-2+…+aA-1DL+π0DL-1+π1DL-2+…+πL-2D1+πL-1

yields a remainder equal to 0 when divided by the corresponding CRC generator polynomial.

The bits after CRC attachment are denoted by b0,b1,…,bB-1, where B=A+L. The relation between ak and bk is:

bk=akfor k=0,1,2,…,A-1 bk=πk-Afor  k=A, A+1,…, A+L-1.

## 8.2	Channel coding

The bit sequence for a given block input to channel coding is denoted c0,c1,…, cK-1, where K is the number of bits to encode.

A tail biting convolutional code with constraint length 7 and coding rate 1/3 is defined.

The configuration of the convolutional encoder is presented in Figure 8.2-1.

The initial value of the shift register of the encoder shall be set to the values corresponding to the last 6 information bits in the input stream so that the initial and final states of the shift register are the same. Therefore, denoting the shift register of the encoder by s0,s1,s2,…, s5, then the initial value of the shift register shall be set to

si=cK-1-i.

Figure 8.2-1: Rate 1/3 tail biting convolutional encoder

The encoder output streams dk0, dk1 and dk2correspond to the first, second and third parity streams respectively, as shown in Figure 8.2-1.

## 8.3	Sequence generation

The output pseudo-random m-sequence of length MPN is xk, k=0, 1, …MPN-1.

If MPN=7, the sequence x=1, 0, 0, 1, 1, 1, 0, i.e. in GF(2), xm+3=xm+2+xm, with x0=1, x1=0, x2=0.

If MPN=31, the sequence x=0, 1, 0, 0, 1, 0, 0, 0, 0, 1, 0, 1, 0, 1, 1, 1, 0, 1, 1, 0, 0, 0, 1, 1, 1, 1, 1, 0, 0, 1, 1, i.e. in GF(2), xm+5=xm+3+xm, with x0=0, x1=1, x2=0, x3=0, x4=1.

## 8.4	Modulation mapping

### 8.4.1	OOK

#### 8.4.1.1	OOK for small frequency shift

In case of OOK modulation for small frequency shift factor RSFS, a single element vi is mapped to modulation symbols x according to:

xj+2K=2j-1vi+1-j
j=0, 1
K=0, 1, …, RSFS-1.

#### 8.4.1.2	OOK for R2D

In case of OOK modulation for R2D, each element bi is mapped to a modulation symbol xi according to xi=bi.

### 8.4.2	BPSK

In case of BPSK modulation for small frequency shift factor RSFS, a single element vi is mapped to modulation symbols x according to:

xj+2K=22j-1vi+1-2j
j=0, 1
K=0, 1, …, RSFS-1.

## 8.5	Line encoding

For line encoding, for a single element vi the encoder output is a line codeword x0,x1 according to:

xj = 2j-1vi+1-j, j=0, 1.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-05 | RAN1#120bis | R1-2503148 |  |  |  | First version | 0.1.0 |
| 2025-06 | RAN1#121 | R1-2504962 |  |  |  | Implementation of further agreements including at RAN1#121, and editor’s improvements | 0.2.0 |
| 2025-06 | RAN#108 | RP-251010 |  |  |  | Submitted to RAN for single-step approval. | 1.0.0 |
| 2025-06 | RAN#108 |  |  |  |  | Further to RAN approval, spec under change control for Rel-19 | 19.0.0 |
| 2025-09 | RAN#109 | RP-252635 | 0001 | - | F | Corrections to the Ambient IoT physical layer | 19.1.0 |
| 2025-12 | RAN#110 | RP-253024 | 0002 | - | F | Corrections to the Ambient IoT physical layer | 19.2.0 |