| 3GPP TS 23.632 V0.3.0 (2019-05) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; User Data Interworking, Coexistence and Migration; Stage 2; (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	7

4	System procedures	7

4.1	General	7

4.2	Authentication	7

4.2.1	General	7

4.2.2	Vector Generation in HSS	8

4.2.3	Vector Generation in UDM/ARPF	9

4.2.4	HSS using the Nudr SBI	9

4.3	Mobility Scenarios	10

4.4	T-ADS	10

4.5	P-CSCF Restoration	11

4.6	SMS Support	12

4.6.1	General	12

4.6.2	MT-SMS Routing Information Retrieval	12

4.6.3	MT-SMS Delivery Failure	12

4.6.4	SMS Alerting	14

5	Network Function Service procedures	15

5.1	HSS Services	15

5.1.1	General	15

5.1.2	Nhss_UEAuthentication_Get service operation	15

5.2	UDM Services	15

5.2.1	General	15

5.2.2	Nudm_UE Context Management Restoration Trigger service operation	16

5.2.3	Nudm_UE Context Management_Get service operation	16

5.2.4	Nudm_MT_ProvideDomainSelectionInfo Service	16

5.2.5	Nudm_EventExposure_Subscribe service operation	16

5.2.6	Nudm_EventExposure_Notify service operation	17

5.3	UDR Services	17

Annex A (informative):	Change history	17

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, certain modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

NOTE 1:	The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

NOTE 2:	The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

NOTE 3:	The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

NOTE 4:	The constructions "can" and "cannot" shall not to be used as substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

NOTE 5:	The constructions "is" and "is not" do not indicate requirements.

# Introduction

This clause is optional. If it exists, it is always the second unnumbered clause.

# 1	Scope

The present document defines the Stage 2 architecture, procedures, flows and Network Function Services for User Data Interworking, Coexistence and Migration within the 5G System.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.002: "Network Architecture".

[4]	3GPP TS 23.380: "IMS Restoration Procedures".

[5]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[6]	3GPP TS 33.501: "Security Architecture and Procedures for 5G System".

# 3	Definitions of terms, symbols and abbreviations

This clause and its three clauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GS UDR	5G Unified Data Repository

EPS UDR	EPS User Data Repository

# 4	System procedures

This clause shall describe procedures and Network Function services by end to-end information flows involving communication between HSS and UDM/UDR.

## 4.1	General

Figure 4.1-1 shows the reference architecture for direct UDM-HSS interworking.

Figure 4.1: Architecture for Direct UDM-HSS interworking

For a list of other SBA interfaces see 3GPP TS 23.501 [2].

For a list of Non-SBA interfaces and Network Nodes interfacing the HSS see 3GPP TS 23.002 [3].

The 5GS-UDR (Unified Data Repository) and EPS-UDR (User Data Repository) may be colocated, forming a common repository.

The HSS shall make use of Nudm services as described in clause 5.2.

The UDM shall make use of Nhss services and Nudr services as described is clauses 5.1 and 5.3.

## 4.2	Authentication

### 4.2.1	General

A subscriber's authentication vectors shall be calculated in one single place in the system which is either the UDM/ARPF or the HSS. For a 5G subscriber without EPS subscription this shall be the UDM/ARPF; for an EPS subscriber without 5GS subscription this shall be the HSS. For subscribers with both 5G and EPS subscription two alternative options have been identified: Vector calculation in the UDM/ARPF and vector calculation in the HSS. Subscription data stored in the 5GS-UDR and EPS-UDR indicates which option is to be used for a given subscriber.

A subscriber's authentication subscription data shall be stored in a single repository so that a single sequence number can be maintained for the subscriber.

### 4.2.2	Vector Generation in HSS

When the UDM receives an authentication information Request from the AUSF it shall check (by means of an 5GS-UDR query) whether the subscribed authentication method is 5G_AKA or EAP_AKA_PRIME and if so whether 5G authentication vector generation for the identified subscriber shall be done in the HSS. If so, the UDM shall make use of the Nhss_UEAuthentication Get service operation to retrieve a 5G authentication vector from the HSS.

Figure 4.2-1 shows the scenario where the authentication vector request for a 5G subscriber who also has an EPS subscription is received by the UDM.

Figure 4.2-1: Authentication for 5G subscriber with authentication vector generation in HSS

1.	The UDM receives an Authentication Vector request, containing the identity of the user (SUPI or SUCI). If SUCI is received, the UDM performs SUCI to SUPI de-concealment. For details of the Nudm_UEAuthentication Service see 3GPP TS 23.502 [5] and 3GPP TS 33.501 [6].

2.	The UDM queries the 5GS-UDR using the SUPI to retrieve Authentication Subscription Information. In this scenario the Authentication Subscription Information contains a subscribed authentication method of 5G_AKA or EAP_AKA_PRIME and an indicator indicating that authentication vector generation shall be performed in the HSS.

3.	The UDM uses the Nhss_UEAuthentication Get service operation to retrieve an authentication vector from the HSS. The request contains the IMSI the authentication method and serving network name.

4.	The HSS reads authentication subscription data from the EPS-UDR. This step is omitted if all relevant authentication subscription data are stored locally in the HSS.

5.	The HSS (AuC) calculates the requested authentication vector taking into account the serving network name and authentication method received in step 3 and the authentication subscription information retrieved from the EPS-UDR.

6.	The calculated authentication vector is sent to the UDM.

7.	The HSS updates the EPS-UDR with the new sequence number. This step is omitted if the sequence number is stored locally in the HSS.

8.	The UDM forwards the authentication vector to the AUSF.

### 4.2.3	Vector Generation in UDM/ARPF

tbc

### 4.2.4	HSS using the Nudr SBI

When the HSS receives an S6a-AIR from the MME, it may check (by means of an EPC-UDR query) whether the subscriber has an 5G subscription. If so, the HSS can use of the Nudr_DM_Query Get service operation to retrieve the authentication subscription data from the 5GS UDR and generate the authentication vector.

Figure 4.2.4-1 shows the scenario where the authentication vector request for a 5G subscriber who also has an EPS subscription is received by the UDM.

Figure 4.2.4-1: Authentication for 5G subscriber with EPS subscription

1.	The HSS receives an Authentication Vector request containing the identity of the user (IMSI).

2.	The HSS queries the 5GS-UDR using the IMSI to retrieve Authentication Subscription Information. Since the subscriber is a 5G subscriber the response indicates that the subscriber's authentication information is stored in the 5GS UDR.

NOTE:	Local configuration in the HSS may indicate that authentication subscription data for all subscribers can be obtained from the 5G UDR and thus this step may be omitted.

3.	The HSS uses the Nudr_DM_Query Get service operation to retrieve the authentication subscription data from the 5GS UDR. The request contains the IMSI formatted as a SUPI.

4.	The HSS (AuC) calculates the requested authentication vector taking into account the serving network name and authentication method received in step 1 and the authentication subscription information retrieved from the 5GS-UDR in step 3.

5.	The calculated authentication vector is returned to the MME.

6.	The HSS updates the 5GS-UDR with the new sequence number.

## 4.3	Mobility Scenarios

## 4.4	T-ADS

Figure 4.4-1 shows the scenario where the HSS receives a T-ADS request from the AS via the Sh interface for a subscriber who has a 5GC subscription.

Figure 4.4-1: T-ADS request for 5G subscriber

1.	The HSS receives a T-ADS request from the AS.

2.	The HSS reads subscription data from the EPS-UDR.

3.	The HSS performs T-ADS info retrieval in the EPC. This may include contacting the MME and/or SGSN.

4.	If the HSS detects in step 2 that the user has a 5GC subscription and unless the user is known not to be registered in 5GC, the HSS uses the Nudm_MT ProvideDomainSelection service operation to retrieve T-ADS information.

5.	The UDM reads data from the 5GS-UDR to see whether the registered AMF(s) support IMS voice over PS.

6-7.	If support/non-support of IMS voice is not homogenious for the User in the AMF for 3GPP access, the UDM retrieves domain selection information from the AMF.

8.	The UDM provides the 5G domain selection info to the HSS.

9.	The HSS combines the 5G domain selection info with the EPS domain selection info from step 3 and sends the result to the AS.

## 4.5	P-CSCF Restoration

Figure 4.5-1 shows the scenario where the HSS receives a P-CSCF restoration indication from the S-CSCF via the Cx interface for a subscriber who has a 5GC subscription.

Figure 4.5-1: P-CSCF Restoration for 5G subscriber

1.	The HSS receives a P-CSCF Restoration indication from the S-CSCF.

2.	The HSS reads subscription data from the EPS-UDR.

3-4.	The HSS acknowledges receipt of Cx-SAR and performs S-CSCF restoration in the EPC. For details see 3GPP TS 23.380 [4] clause 5.4.2.1.

5.	If the HSS detects in step 2 that the user has a 5GC subscription and unless the user is known not to be registered in 5GC, the HSS uses the Nudm_UECM P-CSCF-RestorationTrigger service operation to trigger P-CSCF restoration in 5GC. See also 3GPP TS 23.380 [4] clause 5.8.4.

6.	The UDM reads data from the 5GS-UDR to see which NFs have subscribed to P-CSCF Restoration Notifications (implicitly during registration).

7-8.	The UDM notifies all subscribed NFs (AMF, SMF).

9.	The UDM acknowledges receipt of Nudm_UECM RestorationTrigger to the HSS.

## 4.6	SMS Support

### 4.6.1	General

Potential MT-SMS target nodes in the EPS are the MME/MSC and the SGSN registered at the HSS, while in 5GS MT-SMS target nodes are the 3GPP-SMSF and the Non3GPP-SMSF registered at the UDM. For the role of SMS-Router and IP-SM-GW see 3GPP TS 23.040 [x].

### 4.6.2	MT-SMS Routing Information Retrieval

Figure 4.6.2-1 shows the interaction when the SMS-GMSC retrieves routing information from the HSS for MT-SMS delivery.

Figure 4.6.2-1: SMS Routing Info Retrieval

1. The HSS receives a request for routing information from the SMS-GMSC via MAP or S6c.

2. The HSS queries the EPS-UDR via Ud to read the registered MME/MSC, the registered SGSN and the UE-not-reachable flags for MME/MSC, SGSN, 3GppSMSF and Non3GppSMSF.

3. If at least one of the UE-not-reachable flags for 3GppSMSF and Non3GppSMSF is not set and unless the user is known not to be registered in 5GC, the HSS retrieves the registered SMSF addresses (for which the UE-not-reachable flags are not set) from the UDM.

4-5. The UDM retrieves the requested information from the 5GS-UDR.

6. The UDM forwards the retrieved addresses to the HSS.

7. If at least one of the UE-not-reachable flags was not set, the HSS returns all MT-SMS target node addresses for which the UE-not-reachable flag is not set to the SMS-GMSC; otherwise a negative response (Absent Subscriber SM) is sent to the SMS-GMSC and

8. the SMSC address is added to the Message Waiting Data (MWD) stored in the EPS-UDR.

### 4.6.3	MT-SMS Delivery Failure

Figure 4.6.3-1 shows the interaction when the SMS-GMSC sends Report-SM-Delivery-Status to the HSS.

Figure 4.6.3-1: MT-SMS Delivery Failure

1. The HSS receives a Report-SM-Delivery-Status from the SMS-GMSC indicating the MT-SMS target nodes at which MT-SMS delivery was unsuccessful.

2-3. The HSS reads and updates the Message Waiting Data stored in the EPS-UDR.

4. The HSS acknowledges the receipt of the delivery status to the SMS-GMSC.

5. If during step 3 one of the UE-not-reachable flags for 3GppSMSF and Non3GppSMSF were modified from false to true, the HSS subscribes to notification on UE-Reachability for SMS at the UDM, using the Nudm_EE_Subscribe service operation.

6-7. The UDM queries the 5GS-UDR to see whether UE-Reachability has already been subscribed at the registered AMF(s) (i.e. whether URRP indicators are set).

8-9. If not already subscribed, the UDM subscribes to UE-Reachability notification at the AMF(s) using the Namf_EE service.

10-11. The UDM stores the received EE-Subscription in the 5GS-UDR and if steps 8-9 were performed, the UDM sets the relevant URRP indicators in the 5GS-UDR.

12. The UDM acknowledges the subscription to the HSS.

### 4.6.4	SMS Alerting

Figure 4.6.4-1 shows the interaction when the UE becomes available.

Figure 4.6.4-1: SMS Alerting

1. The UDM receives a Notification or Registration from the AMF.

2-3. The UDM queries the 5GS-UDR to see whether any NF (e.g. the HSS) has subscribed to notifications on UE-reachability for SMS.

4. The UDM acknowledges the message received in step 1.

5. If NFs (e.g. the HSS) have subscribed to UE reachability for SMS notification, the UDM notifies all subscribed NFs.

6-7. The UDM updates (clears) the relevant URRP indicator and deletes the (one-time) EE-subscriptions of UE reachability for SMS in the 5GS-UDR.

8. The HSS reads Message Waiting Data from the EPS-UDR.

9-10. The HSS sends Alert-SC to all SMS-GMSCs stored in the MWD.

11. The HSS removes the SMS-GMSCs from the MWD stored in the EPS-UDR and clears the relevant UE-not-reachable flags.

12. The HSS acknowledges receipt of the Notification to the UDM.

# 5	Network Function Service procedures

This clause shall describe Nudm services offered by the UDM to the HSS, Nhss services offered to by the HSS to the UDM and Nudr services offered by the UDR to the HSS.

## 5.1	HSS Services

### 5.1.1	General

The following table illustrates the HSS Services.

Table 5.1.1-1: NF services provided by HSS

| NF service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| UE Authentication | Get | Request/Response | UDM |

### 5.1.2	Nhss_UEAuthentication_Get service operation

Service operation name: Nhss_UEAuthentication_Get

Description: Requester NF gets the authentication vector from HSS. For AKA based authentication, this operation can be also used to recover from synchronization failure situations.

Inputs, Required: IMSI, authentication method, serving network name.

Inputs, Optional: Synchronization Failure indication and related information (i.e. RAND/AUTS).

Outputs, Required: Authentication vector.

Outputs, Optional: None.

## 5.2	UDM Services

### 5.2.1	General

The following table illustrates the UDM Services.

Table 5.2.1-1: NF services provided by UDM

| NF service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| UECM | P-CSCF-RestorationTrigger | Request/Response | HSS |
|  | Get | Request/Response | HSS |
| MT | ProvideDomainSelectionInfo | Request/Response | HSS |
| EventExposure | Subscribe | Subscribe/Notify | HSS |
|  | Notify |  |  |

### 5.2.2	Nudm_UE Context Management Restoration Trigger service operation

Service operation name: Nudm_UECM_P-CSCF-RestorationTrigger

Description: Requester NF (HSS) triggers the UDM to notify all NFs (AMF, SMF) that have previously subscribed to receive notifications for P-CSCF restoration.

Inputs, Required: SUPI.

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

### 5.2.3	Nudm_UE Context Management_Get service operation

See 3GPP TS 23.502 [5] clause 5.2.3.2.4.

### 5.2.4	Nudm_MT_ProvideDomainSelectionInfo Service

Service operation name: Nudm_MT_ProvideDomainSelectionInfo

Description: Provides the UE information for terminating domain selection of IMS voice in 5G to the consumer NF (HSS).

Inputs, Required: SUPI.

Inputs, Optional: None.

Outputs, Required: Success/Failure indication.

Outputs, Optional: Indication of supporting IMS voice over PS Session or not, Time stamp of the last radio contact with the UE, Current RAT type.

### 5.2.5	Nudm_EventExposure_Subscribe service operation

See 3GPP TS 23.502 [5] clause 5.2.3.5.2.

When used by the HSS in MT-SMS delivery failure scenarios (see clause 4.6.3) the subscribe request shall be a one-time subscription for UE-reachability for SMS and shall indicate whether it is applicable to 3GPP access, Non3GPP access or both.

### 5.2.6	Nudm_EventExposure_Notify service operation

See 3GPP TS 23.502 [5] clause 5.2.3.5.4.

When used by the HSS in SMS Alerting scenarios (see clause 4.6.4) the notify request shall indicate whether it is applicable to 3GPP access or Non3GPP access.

## 5.3	UDR Services

The UDM shall make use of the Nudr_DataRepository Query service operations specified in 3GPP TS 23.502 [5] to retrieve Authentication Subscription Data. If the subscribed authentication method is 5G_AKA or EAP_AKA_PRIME, the AuthenticationSubscriptionData shall include an indicator indicating whether for the identified subscriber authentication vector generation is to be performed by the HSS.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-04 | CT4#90 | C4-191319 |  |  |  | Skeleton | 0.1.0 |
| 2019-04 | CT4#90 | C4-191320 |  |  |  | Scope | 0.2.0 |
| 2019-04 | CT4#90 | C4-191321 |  |  |  | Reference Architecture | 0.2.0 |
| 2019-05 | CT4#91 | C4-192135 |  |  |  | Removal of Editor's Note | 0.3.0 |
| 2019-05 | CT4#91 | C4-192402 |  |  |  | P-CSCF Restoration | 0.3.0 |
| 2019-05 | CT4#91 | C4-192403 |  |  |  | T-ADS | 0.3.0 |
| 2019-05 | CT4#91 | C4-192405 |  |  |  | SMS | 0.3.0 |
| 2019-05 | CT4#91 | C4-192462 |  |  |  | Authentication | 0.3.0 |
| 2019-05 | CT4#91 | C4-192098 |  |  |  | Authentication – HSS using Nudr | 0.3.0 |