| 3GPP TR 23.700-52 V0.3.0 (2025-08) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on SEAL data delivery Phase 3; (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	7

4	Architecture requirements	8

5	Key issues	8

5.1	Key issue #1: Data delivery using multicast/broadcast from both 4G system and 5G system	8

5.1.1	Description	8

5.1.2	Open issues	9

5.2	Key issue #2: SEALDD-S and SEALDD-UU user plane	9

5.2.1	Description	9

5.2.2	Open issues	9

5.3	Key Issue #3: Application traffic pattern and SEALDD data delivery optimization	10

5.3.1	Description	10

5.3.2	Open Issues	10

5.x	Key issue #x: <Title>	10

6	Solutions	10

6.1	Mapping of solutions to key issues	10

6.1	Solution #1: SEALDD-S and SEALDD-UU user plane functionality description and protocol stack	11

6.1.1	Solution description	11

6.1.1.1	General	11

6.1.1.2	Protocol stack for SEALDD-UU	12

6.1.1.2.1	User plane protocol stack for SEALDD-UU	12

6.1.1.2.2	Control plane protocol stack for SEALDD-UU	12

6.1.1.3	Protocol stack for SEALDD-S	13

6.1.1.3.1	User plane protocol stack for SEALDD-S	13

6.1.1.3.2	Control plane protocol stack for SEALDD-S	13

6.1.1.4	User plane protocol stack examples for SEALDD-UU and SEALDD-S	14

6.1.1.4.1	General	14

6.1.1.4.2	Recommended protocol(s) for Regular Transmission	14

6.1.1.4.3	Recommended protocol(s) for Data Storage	14

6.1.1.4.4	Recommended protocol(s) for XR Transmission	15

6.1.1.4.5	Recommended protocol(s) for Transmission Quality Measurement	15

6.1.2	Architecture Impacts	15

6.1.3	Corresponding APIs	15

6.1.4	Solution evaluation	15

6.x	Solution #x: <title>	16

6.x.1	Solution description	16

6.x.2	Architecture Impacts	16

6.x.3	Corresponding APIs	16

6.x.4	Solution evaluation	16

7	Overall evaluation	16

8	Conclusions	16

8.1	General conclusions	16

8.2	Conclusions of key issue #x	16

Annex A: Change history	17

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

SEALDD as defined in 3GPP TS 23.433 [2] provides several data delivery service based on the 3GPP network capabilities for the application service provider. In order to cater the various data delivery requirements (e.g., data delivery optimization based on traffic behaviour) of the application service, the SEALDD should continue the evolution and improvement.

This TR provides a thorough study of potential enhancements to SEALDD under the scope provided in clause 1.

# 1	Scope

The present document is to study on potential enhancements to Service Enabler Architecture Layer (SEAL) data delivery (DD). Specifically, the study is to address data delivery using network multicast/broadcast capability from both 4G and 5G, the SEAL-DD user plane (i.e., SEALDD-UU, SEALDD-S) aspects (e.g., protocol stack(s), user plane functionality description) and the data delivery of new application traffic pattern where the application layer interactions are non-continuous and each of the interactions may be independent with each other.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.433: "Service Enabler Architecture Layer for Verticals (SEAL); Data Delivery enabler for vertical applications".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Architecture requirements

Editor's Note:	This clause describes architectural requirements

# 5	Key issues

## 5.1	Key issue #1: Data delivery using multicast/broadcast from both 4G system and 5G system

### 5.1.1	Description

Many services/applications like live broadcast, group communication, and group message delivery have requirements on the one-to-many transmission. Using 3GPP network multicast/broadcast transmission to delivery the one-to-many communication content can help simplify the application DL delivery e.g., only one copy of the content is suffcient.

3GPP system now support both the 4G eMBMS and the 5MBS. However, there are some issues to use the existing mechanisms defined in 3GPP TS 23.247 [23.247] and 3GPP TS 26.502 [26.502]:

-	Too many configurations options, interfaces: Before using 3GPP multicast/broadcast transmission, the VAL service provider needs to be aware of the configuration options listed in the annex A of in 3GPP TS 23.247 [23.247] that the NEF and the MBSF/MBSTF could be optional. Also, the VAL service provider needs to differentiate the transport Only Mode and Service Mode to select the correct interface from a list of interfaces (e.g., the control plane interface including N33, N5, Nmb13, Nmb10, xMB-C, MB2-C) towards the AF. Such complexity makes it very difficult for the VAL service provider to use the 3GPP multicast/broadcast transmission.

-	Application needs to differentiate and handle 4G eMBMS and 5MBS individually. When required both the eMBMS and 5MBS, the VAL server has to create two User services if the network provide the service layer interworking with MBSF and MBSTF as indicated in clause 4.9 of 3GPP TS 26.502 [26.502], or it has to create the eMBMS bearer towards the BM-SC and the MBS session towards the MB-SMF (optionally via the NEF) individually if the network does not provide the service layer interworking which requires the application server themselves to handle the interworking. From the application layer, such two User Services or individual eMBMS bearer and MBS session increases the difficulty of using 3GPP broadcast and multicast.

-	Application has to select the 5MBS session type. The VAL server needs to select the multicast MBS session and the broadcast MBS session. It also requires the VAL service provider to have enough 3GPP MBS background and knowledge to select a proper type.

The NRM defined in 3GPP TS 23.434 [23.434] supports the multicast/broadcast resource control via several different operations of the SS_NetworkResourceAdaptation API.

Table 5.x.1-1 NRM API supporting multicast/broadcast

| Operation Name | Applicable for 5G MBS or 4G eMBMS, or both |
| --- | --- |
| Request_Multicast_Resource | 4G eMBMS only |
| Request_Multicast/Broadcast_Resource Update_Multicast/Broadcast_Resource Delete_Multicast/Broadcast_Resource Activate_Multicast_Resource Deactivate_Multicast_Resource | 5G MBS only |

Even with the NRM SS_NetworkResourceAdaptation API, the VAL server has to handle the 5G MBS or 4G eMBMS individually.

In addition, during the MBS session update as described in clause 14.3.4A.3 of 3GPP TS 23.434 [23.434], new MBS session(s) may be created per each grouped MBS service area, and the NRM returns additional user plane IP address(es) and port for VAL to deliver the DL packet. In this case, the VAL sever needs to maintain several downlink delivery endpoints towards the 3GPP system for the same content. The application layer is burdened with mentioned issues.

Such complexities mentioned above should be hidden from the VAL service provider to promote the usage of 3GPP multicast/broadcast. SEALDD is designed to provide simple data delivery service for the application data. So how to provide a multicast/broadcast data delivery service which hides the complexities e.g., the configuration options, the different 4G eMBMS management (e.g., eMBMS bearer activation) and 5G MBS session management (e.g., 5MB session creation) to the VAL service provider is crucial to the success of 3GPP multicast/broadcast transmission.

### 5.1.2	Open issues

1.	How to design a multicast/broadcast data delivery service which is able to hide the complexities for the VAL server over SEALDD-S (e.g., the configuration options, the different 4G eMBMS management (e.g., eMBMS bearer activation) and 5G MBS session management (e.g., 5MB session creation) session type selection, multiple downlink delivery endpoints towards the 3GPP system for the same content to the VAL service provider)?

## 5.2	Key issue #2: SEALDD-S and SEALDD-UU user plane

### 5.2.1	Description

SEALDD services require both control signalling messages and user plane to support the delivery of application traffic. Currently, in 3GPP TS 23.433 [2] only the control signalling messages over SEALDD-UU and SEALDD-S are specified for SEALDD services. However, SEALDD user plane (i.e., SEALDD-UU, SEALDD-S) aspects are not fully and well specified. For example, the user plane protocol stack(s) is missing.

In addition, the user plane functionalities description is missing even though several user plane functionalities are already described in the specification, including but not limited to:

1)	The SEALDD flow ID is allocated to uniquely identify the application traffic.

2)	The SEALDD enabled data transmission quality measurement in clause 9.7.2 of 3GPP TS 23.433 [2] requires the DL monitoring packet or the dummy DL SEALDD packet.

3)	SEALDD enabled E2E redundant transmission provides the replication and deduplication between the two SEALDD flows carrying application traffic.

In addition, different SEALDD services may use different SEALDD user plane protocols and functionalities to convey the application traffic, e.g., the user plane functionalities used by SEALDD E2E redundant transmission service could be different from the one used by SEALDD regular connection management service.

The overall SEALDD user plane protocol stack information is not specified in 3GPP TS 23.433 [2], which can further assist stage 3 groups to make choice of suitable user plane protocols for supporting SEALDD services.

### 5.2.2	Open issues

1.	The user plane aspects (e.g., user plane protocol stack, user plane functionality description) of SEALDD-S and SEALDD-UU to be used for the SEAL data delivery services should be studied.

## 5.3	Key Issue #3: Application traffic pattern and SEALDD data delivery optimization

### 5.3.1	Description

The application traffic is dynamic and changes over time. For example, the application traffic can be AI-based applications like GenAI apps, live streaming etc. Moreover different application traffic pattern and interaction requires different QoS for a better QoE to the users.

The SEALDD-UU carries the VAL application traffic. The SEALDD server manages the SEALDD-UU (e.g. establishing a redundant connection) and delivers the application traffic over the SEALDD-UU. Currently, the SEALDD server cannot understand the traffic pattern of the VAL app over the SEALDD-UU. It does not know how the VAL app traffic behaves at different times.

Currently, the SEALDD server connection parameters is not based on the different traffic patterns and this may lead to suboptimal configuration of the connection parameters. There are no mechanisms in the SEALDD server to improve the data delivery service with different traffic pattern awareness. Therefore, it becomes necessary for the SEALDD layer to optimize the SEALDD-UU transmission based on different traffic patterns to handle the dynamicity and interaction of the application traffic.

### 5.3.2	Open Issues

1.	How SEALDD server know different SEALDD-UU application traffic patterns (e.g., characterization, differentiation of dynamic application interaction in the SEALDD layer).

2.	How to enable the SEALDD layer (server, client) data transmission to optimize (e.g., QoS provisioning and achieve QoS guarantee) for the different SEALDD-UU traffic patterns.

## 5.x	Key issue #x: <Title>

Editor's Note:	This clause describes the key issue.

Editor's Note:	Please provide a suitable title for the key issue.

# 6	Solutions

## 6.1	Mapping of solutions to key issues

Table 6.1-1: Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #1 |  | X |  |  |
| Sol #2 |  |  |  |  |

## 6.1	Solution #1:	SEALDD-S and SEALDD-UU user plane functionality description and protocol stack

### 6.1.1	Solution description

#### 6.1.1.1	General

Currently, SEALDD layer provide several data delivery services towards the VAL layer with different user plane characteristics as analysed in table 6.1.1.1-1.

Table 6.1.1.1-1: SEALDD delivery service user plane analysis

| Services of SEALDD service | User plane characteristic of SEALDD-S | User plane characteristic of SEALDD-UU |
| --- | --- | --- |
| Regular Transmission | Address: VAL server endpoint address (e.g., IP address and/or port, URL) and SEALDD server endpoint address (e.g., IP address and/or port, URL). | Address: SEALDD server endpoint address (e.g., IP address and/or port, URL) and SEALDD client endpoint address (e.g., UE's addresses/ports). Identifier: SEALDD-UU Flow ID |
| URLLC Transmission | Address: VAL server endpoint address (e.g., IP address and/or port, URL), and SEALDD server endpoint address (e.g., IP address and/or port, URL). | Address: SEALDD server endpoint address (e.g., IP address and/or port, URL) and SEALDD client endpoint address (e.g., UE's addresses/ports). Identifier: SEALDD-UU Flow ID shared by the two paths. |
| Data Storage | Put DL data to the SEALDD server (storage server) - Push mode in a standalone user plane Fetch data from the SEALDD server - Fetch mode in a standalone user plane | Put UL data to the SEALDD server (storage server) - Push mode in a standalone user plane Fetch data from the SEALDD server - Fetch mode in a standalone user plane |
| BAT data transfer | Refers to the Data storage | Refers to the Data storage |
| XR Transmission | Address: VAL server endpoint address (e.g., IP address and/or port, URL) and SEALDD server endpoint address (e.g., IP address and/or port, URL). | Address: SEALDD server endpoint address (e.g., IP address and/or port, URL) and SEALDD client endpoint address (e.g., UE's addresses/ports). Identifier: SEALDD-UU Flow IDs and multi-modal flow ID. |
| Transmission Quality Measurement |  | SEALDD UU DL monitoring header, or dummy DL SEALDD packet generated by SEALDD server time stamp: local time T1, T2, T3, T4 |

Editor's note:	This table may be not complete and any further updates is FFS.

#### 6.1.1.2	Protocol stack for SEALDD-UU

##### 6.1.1.2.1	User plane protocol stack for SEALDD-UU

This clause illustrates the protocol stack for the user plane data delivery related with SEALDD-UU.

Figure 6.1.1.2.1-1: SEALDD-UU user plane stack

SEALDD-UU user plane:	This user plane of the SEALDD-UU layer corresponds to the protocol data unit carried between the SEALDD client and the SEALDD server over the SEALDD-UU flow. The protocol(s) used for this SEALDD-UU may varies due to different SEALDD data delivery service.

N6 protocol stack:	This set of protocols/layers depends on the UPF.

NOTE 1:	For simplicity, only a box named intermediate protocol stacks are shown. For details, it refers the user plane protocol stacks as described in 3GPP TS 23.501 [23.501].

The user plane of SEALDD-UU layer shall support the functions/characteristics listed in table 6.1.1.1.

##### 6.1.1.2.2	Control plane protocol stack for SEALDD-UU

This clause illustrates the protocol stack for the control plane data delivery related with SEALDD-UU.

Figure 6.1.1.2.2-1: SEALDD-UU control plane stack

SEALDD-UU control plane: The control plane of the SEALDD-UU layer corresponds to the protocol data unit delivered between the SEALDD client and the SEALDD server. This protocol data unit is used for managing SEALDD-UU user plane (e.g., regular transmission connection establishment, transmission quality measurement, etc.), as per the procedures defined in 3GPP TS 23.433 [2].

N6 protocol stack:	This set of protocols/layers depends on the UPF.

NOTE 1:	For simplicity, only a box named intermediate protocol stacks are shown. For details, it refers the control plane protocol stacks as described in 3GPP TS 23.501 [23.501].

#### 6.1.1.3	Protocol stack for SEALDD-S

##### 6.1.1.3.1	User plane protocol stack for SEALDD-S

This clause illustrates the protocol stack for the user plane data delivery related with SEALDD-S.

Figure 6.1.1.3.1-1: SEALDD-S user plane stack

SEALDD-S user plane:	This user plane of SEALDD-S layer corresponds to the protocol data unit carried between the VAL server and the SEALDD server over the SEALDD-S flow. The protocol(s) used for this SEALDD-S user plane may varies due to different SEALDD data delivery service.

The SEALDD-S-U layer shall support the functions/characteristics listed in table 6.1.1.1.

-

##### 6.1.1.3.2	Control plane protocol stack for SEALDD-S

This clause illustrates the protocol stack for the control plane data delivery related with SEALDD-S.

Figure 6.1.1.3.2-1: SEALDD-S control plane stack

SEALDD-S control plane: The control plane of the SEALDD-S layer corresponds to the protocol data unit delivered between the VAL server and the SEALDD server. This protocol data unit is used for managing SEALDD-S user plane data transmission, etc., as per the procedures defined in 3GPP TS 23.433 [2].

#### 6.1.1.4	User plane protocol stack examples for SEALDD-UU and SEALDD-S

##### 6.1.1.4.1	General

This clause illustrates the recommended protocol(s) for different SEALDD data delivery service related to SEALDD-UU and SEALDD-S user plane.

NOTE:	This clause could serve as the basis for the informative Annex of the TS.

##### 6.1.1.4.2	Recommended protocol(s) for Regular Transmission

Regular transmission supports the data delivery of live streaming. The VAL server (content provider) sends live streaming data to the SEALDD server via RTMP/SRT protocol. The SEALDD server handles all Stream Processing, such as converting the RTMP stream into protocols compatible with various playback devices – including RTMP, HTTP-FLV, or HLS. The SEALDD server may then deliver the data to the VAL UE (player) where the SEALDD client resides, using RTMP, HTTP-FLV, or HLS protocols.

Figure 6.1.1.4.2-1: The recommend protocol(s) for live streaming delivery in SEALDD-UU and SEALDD-S user plane stack

Regular transmission supports the VOD. The VAL server (content provider) could send VOD data to the SEALDD server via HTTP(S) “pull” mode. The SEALDD server handles all Stream Processing, such as converting the HTTP(S) stream into protocols compatible with various playback devices – including HLS, HTTP-FLV, or DASH. The SEALDD server may then deliver the data to the VAL UE (player) where the SEALDD client resides, using HLS, HTTP-FLV, or DASH protocols.

Figure 6.1.1.4.2-2: The recommended protocol(s) for VOD in SEALDD-UU 
and SEALDD-S user plane stack

##### 6.1.1.4.3	Recommended protocol(s) for Data Storage

In the data storage service creation procedure, the VAL server/SEALDD client (content provider) transmits data to be stored to the SEALDD server via HTTP/HTTPS protocol. After authorized verification, the SEALDD server configures the storage service for the request and stores data on the storage server. The SEALDD server then sends a response to the SEALDD client/VAL server via the HTTP/HTTPS protocol.

Figure 6.1.1.4.3-1: The recommended protocol(s) for Data Storage in 
SEALDD-UU and SEALDD-S user plane stack

##### 6.1.1.4.4	Recommended protocol(s) for XR Transmission

XR Transmission supports the data delivery of real-time streaming media. The VAL server (content provider) transmits real-time media data to the SEALDD server via WebRTC protocol. The SEALDD server manages all Stream Processing, including converting the WebRTC data into playback-compatible protocols such as RTMP/HLS/FLV. Subsequently, the SEALDD server delivers processed data to the VAL UE (player) hosting the SEALDD client using the RTMP/HLS/FLV protocol.

Figure 6.1.1.4.4-1: The recommended protocol(s) for XR Transmission 
in SEALDD-UU and SEALDD-S user plane stack

##### 6.1.1.4.5	Recommended protocol(s) for Transmission Quality Measurement

Transmission Quality Measurement supports monitoring of transmission quality in an on-going regular data transmission connection. The monitoring information can be carried in the protocol extension header or in the protocol payload. Accordingly, the recommended Transmission Quality Measurement protocol depends on the protocol employed by the monitored data transmission connection.

### 6.1.2	Architecture Impacts

This solution has no architecture impacts.

### 6.1.3	Corresponding APIs

This solution has no new API.

### 6.1.4	Solution evaluation

Editor's note:	This clause describes an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

## 6.x	Solution #x: <title>

Editor's Note:	Please provide a suitable title for the solution.

### 6.x.1	Solution description

Editor's Note:	This clause describes the solution. Each solution should clearly describe which of the key issues it covers and how.

### 6.x.2	Architecture Impacts

Editor's note:	This clause describes the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 6.x.3	Corresponding APIs

Editor's note:	This clause describes the corresponding APIs for supporting the solution.

### 6.x.4	Solution evaluation

Editor's note:	This clause describes an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

# 7	Overall evaluation

Editor's Note:	This clause describes evaluation of different solutions.

# 8	Conclusions

## 8.1	General conclusions

Editor's note:	This clause describes general conclusions for the study.

## 8.2	Conclusions of key issue #x

Editor's Note:	This clause describes conclusions for the specific key issue.

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-04 | SA6#66 | S6-251383 |  |  |  | Skeleton | 0.0.0 |
| 2025-04 | SA6#66 |  |  |  |  | Implemented the following pCRs, S6-251384, S6-251386, S6-251467, S6-251483 | 0.1.0 |
| 2025-06 | SA6#67 |  |  |  |  | Implemented the following pCR, S6-252407 | 0.2.0 |
| 2025-08 | SA6#68 |  |  |  |  | Implemented the following pCRs, S6-253622, S6-253623 | 0.3.0 |