| 3GPP TR 23.700-86 V0.4.0 (2022-09) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Architecture Enhancement to support Ranging based services and sidelink positioning (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	8

1	Scope	10

2	References	10

3	Definitions of terms and abbreviations	11

3.1	Terms	11

3.2	Abbreviations	12

4	Architecture assumptions and requirements	12

4.1	Architecture assumptions	12

4.2	Architecture requirements	12

4.3	General reference architecture	13

4.3.1	General	13

4.3.2	Functional descriptions	16

5	Key Issues	17

5.1	Key Issue #1: Support of authorisation and policy/parameter provisioning to UE	17

5.1.1	General description	17

5.2	Key Issue #2: Ranging service operation procedure with the assistance of another UE	18

5.2.1	General description	18

5.3	Key Issue #3: Ranging/Sidelink Positioning device discovery	18

5.3.1	General description	18

5.4	Key issue #4: Control of Operations for Ranging/Sidelink positioning	19

5.4.1	General description	19

5.5	Key Issue #5: Network assisted Sidelink Positioning for In Network Coverage and Partial Network Coverage	20

5.5.1	General description	20

5.6	Key Issue #6: Ranging and sidelink positioning service exposure to a UE	20

5.6.1	General description	20

5.7	Key Issue #7: Ranging/Sidelink Positioning service exposure to Application server and for network assisted sidelink positioning	21

5.7.1	General description	21

5.8	Key Issue #8: Service Authorization to NG-RAN	21

5.8.1	General description	21

6	Solutions	21

6.0	Mapping of solutions to key issues	21

6.1	Solution #1: Authorisation and policy/parameter provisioning to UE and NG-RAN	22

6.1.1	General	22

6.1.2	Functional descriptions	23

6.1.3	Procedures	24

6.1.4	Impacts on services, entities, and interfaces	24

6.2	Solution #2: V2XP/ProSeP enhancement for the support of Ranging/SL Positioning	24

6.2.1	General	24

6.2.2	Functional descriptions	24

6.2.3	Procedures	26

6.2.3.1	Support of Ranging/SL Positioning for V2X service	26

6.2.3.1.1	PCF based Service Authorization and Provisioning to UE	26

6.2.3.1.2	Procedure for UE triggered V2X Policy provisioning	27

6.2.3.1.3	Ranging/SL Positioning Policy enforcement at UE	27

6.2.3.2	Support of Ranging/SL Positioning for commercial and public safety service	27

6.2.3.2.1	PCF based Service Authorization and Provisioning to UE	27

6.2.3.2.2	Procedure for UE triggered ProSe Policy provisioning	28

6.2.3.2.3	Ranging/SL Positioning Policy enforcement at UE	28

6.2.4	Impacts on services, entities, and interfaces	28

6.3	Solution #3: Solution for direct ranging operation	29

6.3.1	General	29

6.3.2	Procedures	29

6.3.3	Impacts on services, entities, and interfaces	30

6.4	Solution #4: Ranging devices discovery and ranging procedure	30

6.4.1	Description	30

6.4.2	Procedures of Ranging/Sidelink Positioning for 5G ProSe	31

6.4.2.1	General	31

6.4.2.2	Procedures of Ranging/Sidelink Positioning between 2 UEs with Model A discovery	32

6.4.2.3	Procedures of Ranging/Sidelink Positioning between 2 UEs with Model B discovery	33

6.4.2.4	RSPP procedures to exchange the coordination & configuration information	33

6.4.2.4.1	RSPP procedures to exchange Ranging capability	33

6.4.2.4.2	RSPP procedures to exchange Ranging assistant data	34

6.4.2.5	RSPP procedures to exchange Ranging/Sidelink Positioning measurement results	34

6.4.3	Impacts on Existing Nodes and Functionality	34

6.5	Solution #5: Solution to support Ranging/Sidelink Positioning	35

6.5.1	General	35

6.5.2	Functional Descriptions	35

6.5.3	Ranging/ Sidelink Positioning Procedures	35

6.5.4	Impacts on services, entities, and interfaces	36

6.6	Solution #6: Network assisted Ranging and Sidelink Positioning	36

6.6.1	Description	36

6.6.2	Procedures of Located UE discovery	36

6.6.3	Network assisted Ranging and Sidelink Positioning	37

6.6.3.1	Located UE requested procedure on behalf of the Target UE	37

6.6.3.2	Target UE requested procedure	38

6.6.4	Impacts on Existing Nodes and Functionality	39

6.7	Solution #7: Network based UE Sidelink Positioning	39

6.7.1	Introduction	39

6.7.2	Functional Description	39

6.7.3	Procedures	40

6.7.3.1	Network based UE Sidelink Positioning estimation (MT-LR).	40

6.7.4	Impacts on services, entities and interfaces	41

6.8	Solution #8: Discovery of a Located UE for Enhancing Uu based Positioning in Network Coverage	41

6.8.1	Introduction	41

6.8.2	Functional Description	42

6.8.3	Procedures	42

6.8.4	Impacts on services, entities, and interfaces	44

6.9	Solution #9: Solution for KI#6 on Ranging/SL positioning service exposure to UE	45

6.9.1	General	45

6.9.2	Functional descriptions	45

6.9.3	Procedures	46

6.9.3.1	UE involved in SL ranging consumes the Ranging/SL positioning service	46

6.9.3.2	3rd party UE consumes the Ranging/SL positioning service	47

6.9.3.2.1	General	47

6.9.3.2.2	Service exposure to the 3rd party UE via PC5	47

6.9.3.2.3	Service exposure to the 3rd party UE via 5GC/Uu (network based)	48

6.9.4	Impacts on services, entities, and interfaces	49

6.10	Solution #10: Ranging service exposure to UE	50

6.10.1	Description	50

6.10.2	Procedures of Ranging service exposure to SL Positioning Client UE	51

6.10.2.1	Procedures of Ranging service exposure to SL Positioning Client UE	51

6.10.2.2	Deferred procedures of Ranging service exposure to the SL Positioning Client UE	52

6.10.3	Impacts on Existing Nodes and Functionality	52

6.11	Solution #11: UE using exposure API	53

6.11.1	General	53

6.11.2	Functional Description	53

6.11.3	Procedures	54

6.11.3.1	UE access to Ranging and sidelink positioning service exposure.	54

6.11.4	Impacts on services, entities and interfaces	55

6.12	Solution #12: LCS-based Ranging Service Exposure	55

6.12.1	General	55

6.12.2	Functional descriptions	55

6.12.3	Procedures	56

6.12.4	Impacts on services, entities, and interfaces	58

6.13	Solution #13: Ranging/SL positioning service initiated by application server or 5GC NF	58

6.13.1	General	58

6.13.2	Functional descriptions	58

6.13.3	Procedures	59

6.13.3.1	Service initiated by application server	59

6.13.3.2	Service initiated by 5GC NF	60

6.13.4	Impacts on services, entities, and interfaces	61

6.14	Solution #14: General reference architecture for Sidelink positioning and ranging-based services	61

6.14.1	General	61

6.14.2	Functional descriptions	64

6.14.3	Procedures	65

6.14.4	Protocol stack	66

6.15	Solution #15: Service Authorization to NG-RAN for Ranging/Sidelink Positioning	67

6.15.1	General	67

6.15.2	Functional descriptions	67

6.15.3	Procedures	68

6.15.4	Impacts on services, entities, and interfaces	68

6.16	Solution #16: Solution for ranging operation with assistant UE(s)	69

6.16.1	General	69

6.16.2	Functional descriptions	69

6.16.3	Procedures	69

6.16.4	Impacts on services, entities, and interfaces	70

6.17	Solution #17: Ranging/Sidelink positioning with the assistance of assistant UE	70

6.17.1	General	70

6.17.2	Functional descriptions	71

6.17.3	Procedures	72

6.17.4	Impacts on services, entities, and interfaces	73

6.18	Solution #18: Network triggered ranging device discovery	74

6.18.1	Description	74

6.18.2	Control plane-based Network triggered ranging device discovery	74

6.18.2.1	General	74

6.18.2.2	One-shot Control plane-based Network triggered ranging device discovery	75

6.18.2.3	Deferred Control plane-based Network triggered ranging device discovery	76

6.18.3	Application layer-based Network triggered ranging device discovery	77

6.18.3.1	General	77

6.18.3.2	One shot Application layer-based Network triggered ranging device discovery	77

6.18.3.3	Deferred Application layer-based Network triggered ranging device discovery procedure	78

6.18.4	Impacts on Existing Nodes and Functionality	78

6.19	Solution #19: Support of Sidelink Positioning and Ranging signalling over different PC5 RATs	79

6.19.1	General	79

6.19.2	Functional descriptions	79

6.19.2.1	Upper Protocol Layers	79

6.19.2.2	Support of V2X UE operations	82

6.19.2.3	Support of 5G ProSe UE operations	83

6.19.3	Procedures	83

6.19.4	Impacts on services, entities, and interfaces	84

6.20	Solution #20: Network assisted Sidelink positioning with scheduled location time	84

6.20.1	General	84

6.20.2	Functional descriptions	84

6.20.3	Procedures	85

6.20.3.1	MT-LR procedure for Network assisted Sidelink positioning	85

6.20.3.2	MO-LR procedure for Network assisted Sidelink positioning	86

6.20.4	Impacts on services, entities, and interfaces	86

6.21	Solution #21: Network assisted sidelink positioning for partial coverage	87

6.21.1	General	87

6.21.2	Functional descriptions	87

6.21.3	Procedures	88

6.21.4	Impacts on services, entities, and interfaces	89

6.22	Solution #22: Combine Uu positioning capability and/or Ranging/SL positioning capability for the Ranging/SL positioning service	89

6.22.1	General	89

6.22.2	Functional descriptions	90

6.22.3	Procedures	90

6.22.4	Impacts on services, entities, and interfaces	91

6.23	Solution #23: Ranging service exposure to server	91

6.23.1	Description	91

6.23.2	Procedures of NW assistant Ranging service exposure to server	92

6.23.3	Impacts on Existing Nodes and Functionality	93

6.24	Solution #24: Service initiated by Application server over user plane	93

6.24.1	General	93

6.24.2	Functional descriptions	94

6.24.3	Procedures	94

6.24.4	Impacts on services, entities, and interfaces	94

6.25	Solution #25: Ranging/SL positioning service initiated by SL Positioning Client UE	95

6.25.1	General	95

6.25.2	Functional descriptions	95

6.25.3	Procedures	96

6.25.3.1	PC5 based procedure for Ranging/SL positioning initiated by the SL Positioning Client UE	96

6.25.3.2	Network based procedure for Ranging/SL positioning initiated by SL Positioning Client UE (without involving LMF)	97

6.25.3.3	Ranging/SL positioning Application Layer ID registration	99

6.25.4	Impacts on services, entities, and interfaces	99

6.26	Solution #26: Functional Split and Positioning Protocol for SL Positioning	100

6.26.1	General	100

6.26.2	Functional Split	100

6.26.3	Positioning Protocol	101

6.26.4	Impacts on services, entities, and interfaces	101

6.27	Solution #27: LMF selection for Ranging/Sidelink positioning	102

6.27.1	General	102

6.27.2	Functional descriptions	102

6.27.3	Impacts on services, entities, and interfaces	103

6.28	Solution #28: Using LMF for Ranging/Sidelink positioning	103

6.28.1	General	103

6.28.2	Functional descriptions	103

6.28.3	Procedures	104

6.28.3.1	Procedure of LMF triggered Ranging/sidelink positioning	104

6.28.3.2	Procedure of LMF initiated Ranging/sidelink positioning capability negotiation between UE and LMF	106

6.28.4	Impacts on services, entities, and interfaces	106

6.29	Solution #29: Service continuity of periodic ranging with assistant UE	107

6.29.1	General	107

6.29.2	Functional descriptions	107

6.29.3	Procedures	108

6.29.4	Impacts on services, entities, and interfaces	109

6.30	Solution #30: Ranging service operation with the assistant UE(s)	109

6.30.1	Introduction	109

6.30.2	Functional Description	109

6.30.3	Procedures	110

6.30.4	Impacts on services, entities, and interfaces	111

6.31	Solution #31: Enabling SL positioning for the UE out of coverage	111

6.31.1	General	111

6.31.2	Functional Description	111

6.31.3	Procedures	112

6.31.3.1	Procedure of immediate location request procedure with SL positioning	112

6.31.3.2	Procedure of deferred location request procedure with SL positioning	113

6.31.4	Impacts on services, entities, and interfaces	114

6.32	Solution #32: Involving Multiple Reference UEs for Sidelink positioning	114

6.32.1	Description	114

6.32.2	Procedures	117

6.32.3	Impacts on services, entities and interfaces	124

6.33	Solution #33: Common Ranging/Sidelink positioning service exposure	125

6.33.1	General	125

6.33.2	Functional descriptions	125

6.33.3	Procedures	126

6.33.4	Impacts on services, entities, and interfaces	128

6.X	Solution #x: title of the solution	128

6.X.1	General	128

6.X.2	Functional descriptions	128

6.X.3	Procedures	129

6.X.4	Impacts on services, entities, and interfaces	129

7	Evaluation	129

8	Conclusions	129

Annex A: <Informative annex title >	130

Annex B: Change history	131

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document is to study 5GC architecture enhancements to enable Ranging-based services and sidelink positioning for commercial, V2X and public safety use cases in in-coverage, partial coverage, and out-of-coverage of 5G network, which include:

-	Authorization and policy/parameter provisioning for a UE or a group of UEs;

-	Ranging device discovery and service operation procedures between two UEs, between one UE and multiple UEs or via the assistance of another UE;

-	Ranging and sidelink positioning service exposure to a UE or an Application Server who has requested the service.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 22.855: "Study on Ranging-based Services".

[3]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[4]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[5]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[6]	3GPP TR 38.845: "Study on scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases".

[7]	3GPP TS 22.186: "Enhancement of 3GPP support for V2X scenarios".

[8]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[9]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[10]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

[11]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS)".

[12]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[13]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs".

[14]	3GPP TS 23.501: " System Architecture for the 5G System; Stage 2".

[15]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2 ".

[16]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[17]	3GPP TS 24.587: "Vehicle-to-Everything (V2X) services in 5G System (5GS); Stage 3".

[18]	3GPP TR 23.700-33: "Study on System enhancement for Proximity based Services (ProSe) in the 5G System (5GS); Phase 2".

[19]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[20]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[21]	3GPP TR 38.859: "Study on Expanded and Improved NR Positioning".

[22]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2".

# 3	Definitions of terms and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Ranging: refers to the determination of the distance between two UEs or more UEs and/or the direction of one UE (i.e. Target UE) from another UE (i.e. Reference UE) via PC5 interface.

SL Reference UE: A UE, supporting positioning of target UE, e.g. by transmitting and/or receiving reference signals for positioning, providing positioning-related information, etc. using sidelink.

NOTE 1:	SL Reference UE is understood as "Anchor UE" in RAN WG1 TR 38.859 [21].

NOTE 2:	"Reference UE" mentioned in KIs and Solutions of this TR refers to "SL Reference UE".

Target UE: A UE whose distance, direction and/or position is measured with the support from one or multiple SL Reference UEs using Sidelink in the Ranging based service and Sidelink positioning.

Assistant UE: A UE supporting Ranging/Sidelink Positioning between a SL Reference UE and a Target UE over PC5, when the direct Ranging/Sidelink positioning between the SL Reference UE and Target UE cannot be supported. The measurement/result of Ranging/Sidelink Positioning between the Assistant UE and the SL Reference UE and that between the Assistant UE and the Target UE are determined and used to derive the Ranging/Sidelink Positioning result between Target UE and SL Reference UE.

Located UE: A SL Reference UE of which the location is known or is able to be known using Uu based positioning. A Located UE can be used to determine the location of a Target UE using Sidelink Positioning.

SL Positioning Server UE: A UE offering location calculation, for Sidelink Positioning and Ranging based service. It interacts with other UEsover PC5 as necessary in order to calculate the location of the Target UE. Target UE or SL Reference UE can act as SL Positioning Server UE if location calculation is supported.

SL Positioning Client UE: A third-party UE, other than SL Reference UE and Target UE, which initiates Ranging/Sidelink positioning service request on behalf of the application residing on it.

NOTE 3:	The SL Positioning Client UE does not have to support Ranging/Sidelink positioning capability, but a communication between the SL Positioning Client UE and SL Reference UE/Target UE has to be established, either via PC5 or via 5GC, for the transmission of the service request and the result.

Sidelink Positioning: Positioning UE using PC5 to obtain absolute position, relative position, or ranging information.

Positioning: A functionality, which detects a geographical location and optionally, velocity (of e.g. a mobile terminal).

Networkassisted Operation: Operation of Ranging/Sidelink Positioning with the involvement of 5GC NFs for the service request handling and result calculation.

UE-only Operation: Operation of Ranging/Sidelink Positioning in which the service request handling and result calculation are performed by UE.

NOTE 4:	For UE-only Operation, the communication among UEs are over PC5.

Relative position: An estimate of the UE position relative to other network elements or relative to other UEs.

Editor's note:	Definition on the terminology of Ranging and Sidelink positioning will be aligned with RAN WGs, and will be revisited when there's any conclusion in RAN WGs.

Application Layer ID: an identifier identifying a Ranging/SL Positioning-enabled UE within the context of a specific application, is used by the SL Positioning Client UE to identify the target UE and the SL Reference UE in the service request.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AS layer	Access Stratum layer

LOS	Line of Sight

ProSe	Proximity based Services

SL	Sidelink

V2X	Vehicle-to-Everything

# 4	Architecture assumptions and requirements

## 4.1	Architecture assumptions

-	Ranging-based services and sidelink positioning consists of distance measurement, direction measurement, or both. UE is allowed to request the distance measurement only, direction measurement only, or both in the service.

-	Not all UEs are assumed to be Ranging/Sidelink positioning capable. It is assumed that all Ranging/SL positioning capable UEs are ProSe or V2X capable.

-	For direct communication/discovery related aspects, architecture defined in TS 23.287 [3] and TS 23.304 [4] is used as the basis, and the solutions defined for V2X and ProSe will be reused as much as possible.

-	Licensed, unlicensed and ITS spectrum may be used for PC5. If licensed spectrum is used, it shall be fully under operator control:

-	For commercial ranging based service, both licensed and unlicensed spectrum can be used for PC5.

-	For V2X service, licensed, unlicensed and ITS spectrum can be used for PC5.

-	For Public safety service, licensed spectrum is used for PC5.

Editor's note:	RAN has not concluded whether unlicensed spectrum can be considered for R18. This architecture assumption will be revisited once it is concluded in RAN.

-	Privacy protection and other security aspects will be tasked to SA3, and the related impact to architecture enhancement will be based on SA3 conclusion.

## 4.2	Architecture requirements

The enhancement to 5GC architecture shall be able to support Ranging-based services and sidelink positioning in in-coverage, partial coverage and out of network coverage scenarios as defined in TS 22.261 [5] and TR 38.845 [6]:

-	When the UE operates a use case having the corresponding positioning requirements, the requirements should be fulfilled when the UE is inside the network coverage as well as when it is outside the network coverage.

The enhancement to 5GC architecture shall be able to support requirements defined in TS 22.261 [5] to enable Ranging-based services and sidelink positioning, specifically the following:

-	support energy efficient UE ranging operation.

NOTE 1:	Solutions to support low power consumption UEs are developed in RAN WGs, alignment for 5GC impact will be considered later.

-	support for a UE to discover other UEs supporting ranging.

-	provide mechanisms for a MNO, or authorized 3rd party, to provision and manage ranging operation and configurations.

-	support mechanisms for a UE to assist another UE to perform ranging of a third UE.

NOTE 2:	It cannot be assumed that all ranging UEs support the same application for exchange of information.

-	allow ranging service between 2 UEs triggered by and exposed to a third UE.

-	allow ranging service between 2 UEs triggered by and exposed to the application server.

-	support one UE initiating ranging to the other UE.

-	support ranging between UEs belonging to different PLMNs.

-	allow roaming UEs to perform ranging.

The enhancement to 5GC architecture shall be able to support requirements defined in TS 22.186 [7] to enable Ranging-based services and sidelink positioning, specifically the following:

-	support relative position between UEs supporting V2X application.

Both Ranging-based services and sidelink positioning should be based on common architecture.

## 4.3	General reference architecture

### 4.3.1	General

Figure 4.3.1-1 shows a reference architecture for Sidelink Positioning and Ranging-based services for non-roaming operation. In this case, the UE A and UE B that are involved in Sidelink Positioning and Ranging-based services have subscription from the same PLMN. The reference architecture also supports the case that UE-A or UE-B or both are not registered to the network or not in coverage. UE C and UE D may be out of coverage, or with partial network coverage. For simplicity, this figure only shows target UE and reference UE (i.e. UE-A, UE-B, UE-C and UE-D), and there could also be Assistant UE, Located UE and SL Positioning Server UE in the architecture as per the solution.

NOTE 1:	Other 5GC entities not marked with the SL positioning/Ranging label may still need to be involved in SL positioning/Ranging.

Figure 4.3.1-1 Reference architecture for Sidelink Positioning and Ranging-based services for non-roaming and same PLMN operation

Figure 4.3.1-2 shows a reference architecture for Sidelink Positioning and Ranging-based services for Inter-PLMN non-roaming operation. In this case, the UE A and UE B have subscriptions from different PLMNs, i.e. PLMN-A and PLMN-B respectively. UE C and UE D may be out of coverage, or with partial network coverage. For simplicity, this figure only shows target UE and reference UE (i.e. UE-A, UE-B, UE-C and UE-D), and there could also be Assistant UE, Located UE and SL Positioning Server UE in the architecture as per the solution.

NOTE 2: Other 5GC entities not marked with the SL positioning/Ranging label may still need to be involved in SL positioning/Ranging.

Figure 4.3.1-2 Reference architecture for Sidelink Positioning and Ranging-based services for inter-PLMN operations

Figure 4.3.1-3 shows a reference architecture for Sidelink Positioning and Ranging-based services for Inter-PLMN non-roaming operation. In this case, the UE A and UE B have subscriptions from different PLMNs, i.e. PLMN-A and PLMN-B respectively. In addition, UE A is roaming and registered to PLMN-C. UE C and UE D may be out of coverage, or with partial network coverage. For simplicity, this figure only shows target UE and reference UE (i.e. UE-A, UE-B, UE-C and UE-D), and there could also be Assistant UE, Located UE and SL Positioning Server UE in the architecture as per the solution.

NOTE 3:	Other 5GC entities not marked with the SL positioning/Ranging label may still need to be involved in SL positioning/Ranging.

Figure 4.3.1-3 Reference architecture for Sidelink Positioning and Ranging-based services for inter-PLMN operation with roaming

Editor's note:	Whether the architecture(s) for (S)NPN deployments will be considered is FFS.

### 4.3.2	Functional descriptions

Sidelink Positioning and Ranging-based services are supported based on the architectures in clause 4.3.1, with the following reference points:

-	SR1: The reference point between the UE Sidelink (SL) Positioning and Ranging function in the UE and the SL Positioning/Ranging Server. This reference point is out of scope of this specification. It may be used for the configuration and application layer signaling.

-	SR5: The reference point between the Sidelink (SL) Positioning and Ranging function in UEs. It is carried over the PC5 reference point.

Editor's note:	Whether SR5 will be over PC5-RRC, PC5-D, PC5-U and/or PC5-S will be aligned with RAN WGs.

-	PC5: The reference point between the UEs. It also supports the Sidelink Positioning and Ranging operation defined by RAN WGs.

NOTE: Whether it includes the LTE based PC5 and/or NR based PC5 will be addressed by solutions.

Editor's note:	Reference to RAN WG specifications, e.g. TS 38.300 [8], to be added when it is available.

-	N1: In addition to the relevant function defined in TS 23.501 [14], if the UE is in coverage, it may be also used to convey the SL Positioning and Ranging policy (SLPRP) (including service authorization) from AMF to the UE, and to convey the UE's capability from the UE to the AMF. If the LMF supports Sidelink Positioning and Ranging-base services, it may be also used to carry the signaling between UE and LMF, as defined in TS 23.273 [11].

-	N2: In addition to the relevant functions defined in TS 23.501 [14] for N2, in the case of SL Positioning and Ranging-based service is supported by NG-RAN, it is also used to convey the SLPRP policy and parameters (including service authorization) from AMF to NG-RAN.

-	Uu: The reference point between the UE and the NG-RAN.

Following service-based interfaces are used to support Sidelink Positioning and Ranging-based services:

Nlmf:	In addition to the relevant services defined in TS 23.273 [11], if the LMF support SL Positioning and Ranging-based service, it may be used to provide service to other NFs related to it.

Nudm:	In addition to the relevant services defined in TS 23.501 [14] for Nudm, in the case of SL Positioning and Ranging-based service, services provided by UDM are used to get the related subscription information to AMF during Initial registration procedure or UE Configuration Update (UCU) procedure to inform AMF subscription information has changed.

Npcf:	In addition to the relevant services defined in TS 23.501 [14] for Npcf, in the case of SL Positioning and Ranging-based service, services provided by H-PCF are used to provide SL Positioning and Ranging Service related parameters to V-PCF for UE and NG-RAN in the roaming case.

Nudr:	In addition to the relevant services defined in TS 23.501 [14] for Nudr, in the case of SL Positioning and Ranging-based service, services provided by UDR are used to notify the PCF and the UDM of the update of the SL Positioning and Ranging-based service related information.

Namf:	In addition to the relevant services defined in TS 23.501 [14] for Namf, in the case of SL Positioning and Ranging-based service, services provided by AMF are consumed by PCF to provide the SL Positioning and Ranging-based service related parameters for the UE and the NG-RAN to AMF, and to enable the AMF create or update UE context related to SL Positioning and Ranging-based service.

# 5	Key Issues

## 5.1	Key Issue #1: Support of authorisation and policy/parameter provisioning to UE

### 5.1.1	General description

A Ranging/SL positioning capable UE needs necessary configuration data to perform ranging operations, which contains the policies and/or parameters related to the service. Additionally, whether the service is permitted shall also be authorised.

In order to enable Ranging based service and Sidelink positioning over PC5, the following aspects related to authorisation and policy/parameter provisioning need to be studied:

-	Study the principle, and identify the necessary and additional information for service authorisation and (pre-)configuration/provisioning to the UE based on what is specified in clause 5.1.2.1 of TS 23.287 [3], and in clauses 5.1.1, 5.1.2.1, and 5.1.3.1 of TS 23.304 [4].

-	Study the procedures related to service authorisation and policy/parameter (pre-)configuration/provisioning to a UE based on what is specified in clause 6.2 of TS 23.287 [3] and clause 6.2 of TS 23.304 [4].

NOTE:	Architectural reference model as documented in clause 4.2 of TS 23.304 [4] and clause 4.2 of TS 23.287 [3] will be used as baseline for this key issue.

## 5.2	Key Issue #2: Ranging service operation procedure with the assistance of another UE

### 5.2.1	General description

The ranging service operation between two UEs may need the assistance of one or more UEs (e.g. assistant UE) if the direct ranging/sidelink positioning between a reference UE and a target UE cannot be supported, or if the ranging/sidelink positioning is not sufficiently accurate. The direct communication between a reference UE and one or more assistant UE and between a target UE and one or more assistant UEs are respectively supported, so that the assistant UEs can help on measurement and data transmission.

This key issue addresses how to enhance the 5GS to support ranging service operation procedures with the assistance of other UEs (e.g. assistant UEs).

At least the following aspects need to be considered in potential solutions:

-	How to discover and determine one or more assistant UEs for ranging service?

-	What parameters need to be exchanged between different UEs (reference UE, assistant UEs, target UE) during ranging service operation procedures?

-	How the reference UE or target UE gets the ranging result with the assistance of one or more assistant UEs during the ranging service operation procedures?

NOTE:	The direct discovery procedure between reference UE and assistant UEs and between target UE and assistant UEs will be developed under the key issue related to ranging direct discovery.

## 5.3	Key Issue #3: Ranging/Sidelink Positioning device discovery

### 5.3.1	General description

This KI maps to WT 1.3 of the SID. This KI addresses both UE triggered and network triggered Ranging/ Sidelink Positioning.

In most of the cases, the 2 or more UE(s) in a Ranging based service and sidelink positioning cannot always be aware of each other even when they are in proximity.

To enable a Reference observer UE and a target UE to be able to perform measurement and communicate for delivering measurement data, the UEs have to be discovered by each other first.

The discovery procedure is used for a UE to discover or to be discovered by other UE(s) in proximity over the PC5 interface with the aim of performing Ranging and Sidelink Positioning.

Reference UE and Target UE are 2 roles in the Ranging based service and Sidelink Positioning, in terms of who provides the reference plane and reference direction. Either one can be discovered by the other.

Ranging/Sidelink Positioning device discovery may be triggered at the UEs themselves and/or based on network instruction (e.g. under network monitoring, when, within which distance/angle or which UE(s) can be discovered).

Following aspects need to be addressed under this key issue:

1.	How Reference UEs and Target UEs are discovered by each other to perform Ranging based service and Sidelink Positioning for the case of in coverage, partial coverage and out of coverage for both UE triggered and network triggered Ranging/Sidelink Positioning, with the following considerations:

-	How one Target UE and one Reference UE are discovered by each other?

-	How one Target UE and multiple Reference UEs are discovered by each other?

-	How one Reference UE and multiple target UEs are discovered by each other?

NOTE 1:	ProSe direct discovery procedure should be reused as much as possible.

2	How Ranging/Sidelink Positioning devices' discovery is triggered at UE based on network instruction for the cases of in coverage, partial coverage and out of coverage?

NOTE 2:	It is assumed that both the Reference UE(s) and the target UE are Ranging/SL positioning capable.

## 5.4	Key issue #4: Control of Operations for Ranging/Sidelink positioning

### 5.4.1	General description

Ranging and sidelink positioning supports a large variety of use cases, including automotive, public safety, and commercial use. These different uses cases may utilize different access technologies for the communication between the UEs, e.g. LTE based V2X, NR based sidelinks and, in later releases, other RAT technologies may be supported over PC5, e.g. newer version of sidelink and non-3GPP RATs. Therefore, it is beneficial to design a common framework and system to support all these types of operations.

The control signalling part of the Ranging and Sidelink Positioning operation may occur between the UEs before and after the transmission of the Ranging/Sidelink positioning signals. The control signalling includes (but is not limited to):

-	coordination and configuration of the UEs to participate in the ranging/sidelink positioning, e.g. including discovery of the devices, signalling control and configuration information between devices;

-	exchange of measurement results of Ranging/Sidelink positioning signals between the UEs for ranging/Sidelink positioning calculation.

This key issue is focusing on developing the control signalling for Ranging and Sidelink Positioning operations. Specifically, the key issue addresses:

-	What coordination and configuration information are exchanged between the UEs?

-	What are the control signalling procedure to exchange the coordination & configuration information and the Ranging/Sidelink positioning signals measurement results between the UEs?

NOTE 1	Common solution is preferred to be developed if the same control signalling procedure can be supported when the signalling is carried over different RATs in different use cases.

NOTE 2:	Common control signalling protocol is preferred to support the different Ranging/Sidelink positioning signals operation in different spectrums.

NOTE 3:	The solution development may also consider signalling efficiency, overhead, energy efficiency and the robustness.

NOTE 4:	Coordination with RAN WGs is needed to develop an aligned solution.

NOTE 5:	It is desirable to avoid any AS layer impacts to support the control signalling.

-	What are the functionality of upper layers, and what parameters need to be exchanged between the upper layers and AS layer for Ranging/ Sidelink positioning?

## 5.5	Key Issue #5: Network assisted Sidelink Positioning for In Network Coverage and Partial Network Coverage

### 5.5.1	General description

This key issue addresses the case when a UE is not able to perform Uu based positioning estimation. The UE may be out of network coverage, or have network coverage but not able to measure on enough gNBs to perform Uu based positioning. In this case, the UE (i.e. partial coverage or in network coverage) may use Sidelink Positioning to obtain relative positioning with a Located UE which is in network coverage. This case is referred to as Sidelink Positioning in TR 38.845 [6]. In this KI we call this case Network assisted Sidelink Positioning.

For this key issue, the following aspects need to be studied:

-	Discovery of one or more Located UEs with capabilities that offers Sidelink Positioning and/or Uu based positioning to support Network assisted Sidelink Positioning for in network coverage and partial network coverage.

-	Discovery of network capability that offers Network assisted Sidelink positioning for UE in network coverage and partial network coverage.

-	Signalling needed to support Sidelink positioning including both UE-based positioning and network-assisted/based positioning.

NOTE 1:	Security aspects are SA WG3 responsibility, and the related impact to architecture enhancement will be based on SA WG3 conclusion.

NOTE 2:	Coordination with RAN WG is needed for network assisted Sidelink Positioning.

NOTE 3:	Coordination with FS_eLCS_ph3 may be needed if any changes to existing LCS framework.

## 5.6	Key Issue #6: Ranging and sidelink positioning service exposure to a UE

### 5.6.1	General description

The Ranging/Sidelink positioning service between 2 Ranging/Sidelink positioning capable UEs may be exposed to a third UE to obtain the Ranging/Sidelink positioning information between the 2 UEs.

The third UE may request Ranging/Sidelink positioning information immediately, periodically or at a later time.

The service request can also be initiated by one of the Ranging/Sidelink positioning capable UEs providing the service, i.e. Ranging/Sidelink positioning service is exposed to one of the UEs providing the service. In that case, the third UE is the same as one of the UEs providing the service.

The following aspects need to be studied for the potential solutions:

-	How does a Ranging/Sidelink positioning capable UE expose the Ranging/Sidelink positioning service to a UE?

-	What Ranging/Sidelink positioning information are to be exposed to a UE?

-	How to authorize the third UE to consume the Ranging/Sidelink positioning service?

-	How to support Ranging/Sidelink Positioning request?

NOTE:	For direct communication related aspects, V2X and/or ProSe architecture is used as the basis, and V2X and ProSe solutions will be reused as much as possible. Coordination with FS_5G_ProSe_Ph2 study may be required if any impact to R18 ProSe solution is identified.

## 5.7	Key Issue #7: Ranging/Sidelink Positioning service exposure to Application server and for network assisted sidelink positioning

### 5.7.1	General description

Application Server may initiate the Ranging/Sidelink Positioning service request for acquiring Ranging information between 2 UEs. A Ranging/Sidelink Positioning request may also be initiated by 5G network for network assisted sidelink positioning. Application Server/5G network may request Ranging/Sidelink positioning information immediately, periodically or at a later time.

In order to enable Ranging/Sidelink positioning service exposure to Application server and for network assisted measurement management, following aspects need to be studied for the potential solutions:

-	How 5GS exposes the Ranging/Sidelink Positioning measurement management service to 5G network and Application server?

-	What are the architectural enhancements to support Ranging/Sidelink Positioning service exposure to Application server and for network assisted sidelink positioning, the required NF and NF services?

-	How to support Ranging/Sidelink Positioning request?

## 5.8	Key Issue #8: Service Authorization to NG-RAN

### 5.8.1	General description

As defined in TS 38.300 [8], the UE can operate in either Scheduled resource allocation mode or UE autonomous resource selection mode for resource allocation in sidelink. This also applies to Ranging/SL Positioning. If scheduled resource allocation mode is used, service authorization information needs to be provisioned to NG-RAN for the support of Ranging/SL Positioning.

In order to enable Ranging based services and sidelink positioning over PC5, the following aspects related to Service Authorization to NG-RAN need to be studied:

-	What are the service authorization policy/parameter required for the support of Ranging/SL Positioning to be provisioned to NG-RAN and how does 5GC authorize the usage of Ranging based services and sidelink positioning over PC5 and the related policy/parameter?

NOTE:	Procedures for Service Authorization to NG-RAN as documented in clause 6.5 of TS 23.287 [3] and clause 6.6 of TS 23.304 [4] will be used as baseline for this key issue.

# 6	Solutions

## 6.0	Mapping of solutions to key issues

Editor's note:	This clause describes the mapping between solutions and key issues.

Table 6.0-1: Mapping of solutions to key issues

|  | Key Issues |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1 | X |  |  |  |  |  |  | X |
| 2 | X |  |  |  |  |  |  |  |
| 3 |  |  |  | X |  |  |  |  |
| 4 |  |  | X | X |  |  |  |  |
| 5 |  |  | X | X |  |  |  |  |
| 6 |  |  |  |  | X |  |  |  |
| 7 |  |  |  |  | X |  |  |  |
| 8 |  |  |  |  | X |  |  |  |
| 9 |  |  |  |  |  | X |  |  |
| 10 |  | X |  |  |  | X |  |  |
| 11 |  |  |  |  |  | X |  |  |
| 12 |  |  |  |  | X |  | X |  |
| 13 |  |  |  |  |  |  | X |  |
| 14 | X | X | X | X | X | X | X | X |
| 15 |  |  |  |  |  |  |  | X |
| 16 |  | X |  |  |  |  |  |  |
| 17 |  | X |  |  |  |  |  |  |
| 18 |  |  | X |  |  |  |  |  |
| 19 |  |  |  | X |  |  |  |  |
| 20 |  |  |  |  | X |  |  |  |
| 21 |  |  |  |  | X |  |  |  |
| 22 |  |  |  |  |  | X | X |  |
| 23 |  |  |  |  |  |  | X |  |
| 24 |  |  |  |  |  |  | X |  |
| 25 |  |  |  |  |  | X |  |  |
| 26 |  | X | X | X | X | X | X |  |
| 27 |  | X | X | X | X | X | X |  |
| 28 |  | X | X | X | X | X | X |  |
| 29 |  | X |  |  |  |  |  |  |
| 30 |  | X |  |  |  |  |  |  |
| 31 |  |  |  |  | X |  |  |  |
| 32 |  |  |  | X | X |  |  |  |
| 33 |  |  |  |  |  | X | X |  |

## 6.1	Solution #1: Authorisation and policy/parameter provisioning to UE and NG-RAN

Editor's note:	This clause describes a solution addressing one or more key issues identified in clause 5. The structure of the (sub) clauses can be adjusted. The list of key issues which this solution attempts to resolve should be clearly indicated.

### 6.1.1	General

This solution addresses KI#1 and KI#8: Support of authorisation and policy/parameter provisioning to UE, two following major aspects are covered:

-	For the procedures related to Ranging/SL positioning service authorization and policy/parameter provisioning to a UE, only necessary enhancement with what is specified in TS 23.304 [4] clause 6.2, and TS 23.502 [9] clause 4.2.2.2 (Registration Procedure), clause 4.2.4.3 (UE Configuration Update procedure for transparent UE Policy Delivery), clause 4.16.11 (UE Policy Association Establishment procedure), clause 4.16.12 (UE Policy Association Modification procedure), will be documented.

-	Identify necessary information for Ranging/SL positioning service authorization and provisioning based on what is specified in TS 23.304 [4] clauses 5.1.1, 5.1.2.1, and 5.1.3.1.

The PCF based service authorization and provisioning as defined in TS 23.304 [4] are used as baseline for this solution.

### 6.1.2	Functional descriptions

Ranging/SL positioning refers to the determination of the distance between two UEs or more UEs and/or the direction and/or relative positioning of one UE (i.e. Target UE) from another UE (i.e. Reference UE) via PC5 interface. Ranging/SL positioning can be regarded as a new type of function built on top of PC5 mechanism, including PC5 discovery and communication mechanisms. Therefore, identifying Ranging/SL positioning related enhancements compared with legacy PC5 mechanisms are necessary.

The followings reflect the additions provided by the UE w.r.t Ranging/SL positioning:

-	UE includes PC5 Capability for Ranging/SL positioning as part of "5GMM capability" in the Registration Request message. The AMF stores this information for Ranging/SL positioning operation. The PC5 Capability for Ranging/SL positioning indicates whether the UE is capable of Ranging/SL positioning service over PC5 reference point.

-	If the UE does not have valid authorisation and policy/parameters for Ranging/SL positioning, the UE includes the UE Policy Container with indicating the Policy Provisioning Request for Ranging/SL positioning during registration procedure.

-	If the UE indicates the Policy Provisioning Request for Ranging/SL positioning in the UE Policy Container, the PCF determines whether to provision Policy/parameters for Ranging/SL positioning to the UE, as specified in clause 6.1.2.2.2 of TS 23.503 [15], and the PCF provides the Policy/parameters for Ranging/SL positioning to the UE by using the procedure as defined in clause 4.2.4.3 "UE Configuration Update procedure for transparent UE Policy Delivery" in TS 23.502 [9].

The followings reflect the additions of the 5GC processing w.r.t Ranging/SL positioning:

-	If the AMF receives the PC5 Capability for Ranging/SL positioning in the Registration Request message from UE, the AMF selects the PCF which supports Ranging/SL positioning and establishes a UE policy association with PCF for Ranging/SL positioning service Policy/Parameter delivery.

The followings reflect the additions provided to the UE from PCF w.r.t Ranging/SL positioning:

-	A "Ranging/SL positioning service authorized" indication, indicating the UE is authorized to use Ranging/SL positioning service in the PLMN(s).

-	The authorized Ranging/SL positioning service types, authorized application IDs, authorized user IDs.

-	The authorized Ranging/SL positioning parameters.

Editor's note:	the Ranging/SL positioning parameters are FFS and may be coordinated with RAN WGs.

-	The authorized Ranging/SL positioning role, e.g. target UE, reference UE, assistant UE.

-	The authorized discovery mode and UE role in discovery for Ranging/SL positioning.

The followings reflect the additions provided to the NG-RAN from AMF w.r.t Ranging/SL positioning:

-	A "Ranging/SL positioning service authorized" indication based on UE subscription from UDM, indicating the UE is authorized to use Ranging/SL positioning service in the serving PLMN.

-	The authorized Ranging/SL positioning parameters used by the NG-RAN for the resource management of UE's PC5 transmission for Ranging/SL positioning services in network scheduled mode, which is from PCF and stored in AMF.

Editor's note:	the Ranging/SL positioning parameters are FFS and may be coordinated with RAN WGs.

-	The authorized Ranging/SL positioning role based on UE subscription from UDM, e.g. target UE, reference UE, assistant UE.

The parameters may be preconfigured in ME/UICC, the UE can work without provision of configuration parameters from the network, especially in the use case of out-of-coverage. Or when the UE with Ranging/SL positioning capability is in coverage, the UE can access to the network to obtain the necessary configuration data from the network (e.g. PCF). One the other hand, the network will also provide RAN-related configuration data (e.g. Ranging/SL positioning authorized) to RAN node so as to enable RAN node to allocate PC5 resource once UE initiates Ranging/SL positioning operation.

### 6.1.3	Procedures

The procedures specified in TS 23.304 [4] clause 6.2, and TS 23.502 [9] clause 4.2.2.2 (Registration Procedure), clause 4.2.4.3 (UE Configuration Update procedure for transparent UE Policy Delivery), clause 4.16.11 (UE Policy Association Establishment procedure), clause 4.16.12 (UE Policy Association Modification procedure), are applied.

### 6.1.4	Impacts on services, entities, and interfaces

UE:

-	Need to indicate its PC5 capability for Ranging/SL positioning to AMF;

-	Indicate the Policy Provisioning Request for Ranging/SL positioning in the UE Policy Container to PCF.

-	Receive and enforce the Policy and parameter for Ranging/SL positioning.

NG-RAN:

-	Receive and store the UE's Ranging/SL positioning authorization information for network scheduled mode operation.

AMF:

-	Determine whether UE is authorized for Ranging/SL positioning service;

-	Select a PCF capable of authorization Policy and parameter for Ranging/SL positioning service;

-	Forward the UE's Ranging/SL positioning authorization information for network scheduled mode to NG-RAN.

PCF:

-	Determine the UE's Authorization Policy and parameter for Ranging/SL positioning and send it to UE based on the Policy Provisioning Request for Ranging/SL positioning from UE;

-	Send the RAN's Authorization Policy and parameter for Ranging/SL positioning to NG-RAN via AMF.

UDM:

-	Install the new UE subscription information of Ranging/SL positioning services;

## 6.2	Solution #2: V2XP/ProSeP enhancement for the support of Ranging/SL Positioning

### 6.2.1	General

This solution is to address KI#1.

It is assumed that all Ranging/SL positioning capable UEs are ProSe or V2X capable, thus UE will be provisioned with either V2XP or ProSeP together with the Ranging/SL positioning policy/parameter. Some Policy/Parameters of V2XP/ProSeP may also be applied to Ranging/SL Positioning, then it is not necessary to additionally transmit them to the UE when V2XP/ProSeP is also transmitted, thus saves the NG-RAN resources.

Ranging/SL Positioning can be seen as an extended capability of ProSe or V2X.

### 6.2.2	Functional descriptions

Ranging/SL Positioning Policy/Parameters can be categorized into 2 groups:

-	Ranging/SL Positioning specific parameters: These parameters are dedicatedly used for Ranging/SL Positioning, which may contain reused 5G ProSeP/V2XP parameters by defining Ranging/SL Positioning as a specific V2X/5G ProSe service type and the newly defined parameters specifically for Ranging/SL Positioning.

-	ProSeP/V2XP shared parameters: These parameters are used for both V2X/5G ProSe and Ranging/SL Positioning, which contain reused 5G ProSeP/V2XP parameters that can apply to all service types.

The following parameters are defined as Ranging/SL Positioning specific parameters:

-	For Direct Discovery:

-	Parameters used for Direct Discovery (e.g. Destination Layer-2 ID(s) for sending/receiving initial signalling of discovery messages, ProSe identifiers used for Ranging/Sidelink positioning).

-	For Direct Communication:

-	Policy/parameters related to privacy.

-	Policy/parameters when NR PC5 is selected.

-	Authorization policy for Ranging/SL Positioning (new defined).

-	When the UE is "served by NG-RAN":

-	For a Ranging/SL Positioning capable UE, PLMNs it is authorized to perform as a reference UE for Ranging/SL Positioning.

-	For a Ranging/SL Positioning capable UE, PLMNs it is authorized to perform as a target UE for Ranging/SL Positioning.

-	For a Ranging/SL Positioning capable UE, PLMNs it is authorized to perform Ranging/SL Positioning for acquiring relevant distance.

-	For a Ranging/SL Positioning capable UE, PLMNs it is authorized to perform Ranging/SL Positioning for acquiring relevant direction.

-	For a Ranging/SL Positioning capable UE, PLMNs it is authorized to perform high accuracy Ranging/SL Positioning for acquiring relevant distance.

-	For a Ranging/SL Positioning capable UE, PLMNs it is authorized to perform high accuracy Ranging/SL Positioning for acquiring relevant direction.

-	When the UE is "not served by NG-RAN":

-	For a Ranging/SL Positioning capable UE, whether it is authorized to perform as a reference UE for Ranging/SL Positioning.

-	For a Ranging/SL Positioning capable UE, whether it is authorized to perform as a target UE for Ranging/SL Positioning.

-	For a Ranging/SL Positioning capable UE, whether it is authorized to perform Ranging/SL Positioning for acquiring relevant distance.

-	For a Ranging/SL Positioning capable UE, whether it is authorized to perform Ranging/SL Positioning for acquiring relevant direction.

-	For a Ranging/SL Positioning capable UE, whether it is authorized to perform high accuracy Ranging/SL Positioning for acquiring relevant distance.

-	For a Ranging/SL Positioning capable UE, whether it is authorized to perform high accuracy Ranging/SL Positioning for acquiring relevant direction.

The following parameters are defined as ProSeP/V2XP shared parameters:

-	For Direct Discovery:

-	Authorization policy.

-	Radio parameters when the UE is "not served by NG-RAN".

-	Restricted ProSe Discovery UE ID for Restricted Direct Discovery.

-	Group Member Discovery parameters.

-	Validity timer indicating the expiration time of the Policy/Parameter.

-	For Direct Communication:

-	Authorization policy.

-	Groupcast mode 5G ProSe Direct Communication policy/parameters.

-	Radio parameters when the UE is "not served by NR".

Editor's note:	It is FFS if other parameters are needed.

Editor's note:	Whether LTE PC5 related policy/parameters are needed for Ranging/SL positioning control signalling is FFS.

When UE requests Ranging/SL Positioning policy/parameters, it includes the UE Policy Container with indicating Ranging/SL Positioning Policy Provisioning Request in addition to V2X/5G ProSe Policy Provisioning Request. The PCF provides Ranging/SL Positioning policy/parameters in either V2XP or ProSeP to the UE. PCF determines whether to provide the V2XP or ProSeP to the UE depending on whether a V2X Policy Provisioning Request or a 5G ProSe Policy Provisioning Request is indicated together:

-	When V2X Policy Provisioning Request is indicated together with Ranging/SL Positioning Policy Provisioning Request, PCF provides Ranging/SL Positioning policy/parameters in the V2XP;

-	When 5G ProSe Policy Provisioning Request is indicated together with Ranging/SL Positioning Policy Provisioning Request, PCF provides Ranging/SL Positioning policy/parameters in the ProSeP;

-	When both V2X Policy Provisioning Request and 5G ProSe Policy Provisioning Request are indicated together with Ranging/SL Positioning Policy Provisioning Request, PCF provides Ranging/SL Positioning policy/parameters in both the V2XP and the ProSeP.

To support Ranging/SL Positioning, UE enforces either the Ranging/SL Positioning policy/parameters of V2XP or the those in the ProSeP based on the service request:

-	When the service request is for a V2X service, it enforces Ranging/SL Positioning policy/parameters of V2XP;

-	When the service request is for a commercial service or a public safety service, it enforces Ranging/SL Positioning policy/parameters of ProSeP.

### 6.2.3	Procedures

#### 6.2.3.1	Support of Ranging/SL Positioning for V2X service

##### 6.2.3.1.1	PCF based Service Authorization and Provisioning to UE

If the UE supports V2X communication & Ranging/SL Positioning and it does not have valid V2X policy/parameters, the UE includes the UE Policy Container with indicating Ranging/SL Positioning Policy Provisioning Request in addition to the V2X Policy Request during registration procedure.

If the UE indicates Ranging/SL Positioning Policy in addition to the V2X Policy Provisioning Request in the UE Policy Container, the PCF determines to include in the V2XP the following policy/parameters in addition by considering Ranging/SL Positioning as a special V2X service type with the mapping parameters:

-	Policy/parameters for PC5 Tx Profile selection;

-	Policy/parameters related to privacy;

-	Policy/parameters when NR PC5 is selected.

PCF also determines to include Authorization policy for Ranging/SL Positioning as defined in clause 6.2.2 in the V2XP.

The PCF provides the V2XP to the UE by using "UE Configuration Update procedure for transparent UE Policy Delivery" procedure as defined in clause 4.2.4.3 of TS 23.502 [9].

##### 6.2.3.1.2	Procedure for UE triggered V2X Policy provisioning

The UE triggered Policy Provisioning procedure is initiated by the UE to request V2X Policy/Parameter from the PCF when UE determines the V2X Policy/Parameter is invalid in the following cases:

-	if the validity timer indicated in the V2X Policy/Parameter expires;

-	if there are no valid parameters, e.g. for the V2X service type a UE wants to use, for current area, or due to abnormal situation.

Figure 6.2.3.1.2-1: UE triggered V2X Policy provisioning procedure

1.	The UE sends UL NAS TRANSPORT message carrying the UE Policy Container (UE V2X Policy Provisioning Request to request V2X policies) to the AMF.

2.	The AMF sends the Namf_Communication_N1MessageNotify request to the PCF including the UE Policy Container received from UE.

3.	The PCF receives UE Policy Container which indicates UE Policy Provisioning Request to request Ranging/SL Positioning policy/parameters in addition to V2X policy/parameters. The PCF determines the authorised policy/parameters and deliver them to the UE as defined in 6.2.3.1.1.

##### 6.2.3.1.3	Ranging/SL Positioning Policy enforcement at UE

When Ranging/SL Positioning is performed for V2X service, the UE applies parameters received in V2XP for Ranging/SL Positioning.

#### 6.2.3.2	Support of Ranging/SL Positioning for commercial and public safety service

##### 6.2.3.2.1	PCF based Service Authorization and Provisioning to UE

If the UE supports one of 5G ProSe capabilities & Ranging/SL Positioning and it does not have valid 5G ProSe policy/parameters, the UE includes the UE Policy Container with indicating Ranging/SL Positioning Policy Provisioning Request in addition to the 5G ProSe Policy Provisioning Request during registration procedure.

If the UE indicates Ranging/SL Positioning Policy Provisioning Request in addition to the 5G ProSe Policy Provisioning Request in the UE Policy Container, the PCF determines to include in the 5G ProSeP the following policy/parameters in addition to the other policy/parameters by considering Ranging/SL Positioning as a special 5G ProSe service identified by a predefined ProSe Identifier with the mapping parameters:

-	Parameters used for 5G ProSe Direct Discovery.

-	Policy/parameters related to privacy.

-	Policy/parameters when NR PC5 is selected.

PCF also determines to include Authorization policy for Ranging/SL Positioning as defined in clause 6.2.2 in the ProSeP.

The PCF provides the 5G ProSe policy/parameters to the UE by using "UE Configuration Update procedure for transparent UE Policy Delivery" procedure as defined in clause 4.2.4.3 of TS 23.502 [9].

##### 6.2.3.2.2	Procedure for UE triggered ProSe Policy provisioning

The UE triggered Policy Provisioning procedure is initiated by the UE to request ProSe Policy/Parameter from the PCF when UE determines the 5G ProSe Policy/Parameter is invalid in the following cases:

-	if the validity timer indicated in the 5G ProSe Policy/Parameter expires;

-	if there are no valid parameters, e.g. for the 5G ProSe Identifier a UE wants to use, for current area, or due to abnormal situation.

Figure 6.2.3.2.2-1: UE triggered 5G ProSe Policy provisioning procedure

1.	The UE sends the UL NAS TRANSPORT message carrying the UE Policy Container (UE Policy Provisioning Request to request 5G ProSe policies) to the AMF.

2.	The AMF sends Namf_Communication_N1MessageNotify request to the PCF including the UE Policy Container received from UE.

3.	The PCF receives UE Policy Container which indicates UE Policy Provisioning Request to request Ranging/SL Positioning policies in addition to 5G ProSe policies. The PCF determines the authorized policy and deliver them to the UE as defined in clause 6.2.3.2.1.

##### 6.2.3.2.3	Ranging/SL Positioning Policy enforcement at UE

When Ranging/SL Positioning is performed for commercial and public safety service, the UE applies parameters received in 5G ProSe policy for Ranging/SL Positioning.

### 6.2.4	Impacts on services, entities, and interfaces

The solution has impacts in the following entities:

-	UE: differentiates Ranging/SL Positioning for V2X service or for commercial/public safety service to determine whether to apply ProSe or V2XP to enforce Ranging/SL Positioning.

-	PCF: determines whether to provide the V2XP or ProSeP to the UE depending on whether a V2X Policy Provisioning Request or a 5G ProSe Policy Provisioning Request is indicated together; support of new Ranging/SL Positioning policy/parameter.

## 6.3	Solution #3: Solution for direct ranging operation

### 6.3.1	General

This solution resolves Key Issue #4 for Operation controls for Ranging/Sidelink positioning. In particular, this solution tries to solve what coordination and configuration information are exchanged between the UEs, and how to exchange the coordination and configuration information and the ranging results between the UEs.

In general, this solution proposes three steps in the direct Ranging operation:

-	The upper layer (e.g. Ranging layer) performs the ranging parameter coordination between the UEs, and the upper layer provides the ranging configuration to the AS layer. The ranging configuration includes the Ranging role (Reference UE or Target UE), one time or period ranging, ranging for distance or direction measurement or both.

-	The lower layer (e.g. AS layer) transmits or receives Ranging signalling and performs measurements according to the ranging configuration;

-	The UE calculates the ranging results, and the ranging results could be shared between the UEs via the upper layer.

### 6.3.2	Procedures

Figure 6.3.2-1: high-level procedure for Ranging operation control

1.	UE1 and UE2 may get the ranging authorization policy and parameters from PCF during the registration procedure. The ranging authorization policy and parameters may include whether the UE is authorized as Reference UE or Target UE.

2.	When the UE1 gets the ranging request from the application layer, other UE or 5GC NF, UE1 can discovery UE2 by using the solutions for KI#3 Ranging/Sidelink Positioning device discovery.

3.	UE1 and UE2 can perform the PC5 connection establishment, as defined in TS 23.304 [4].

NOTE:	PC5 connection is established for the signalling interaction of ranging parameters between two UEs. This step is optionally needed only when Ranging/Sidelink positioning is between two UEs.

4.	UE1 sends the ranging request to the UE2 to negotiate the ranging parameters, and the ranging request can be a new PC5-S signalling carried by the PC5 connection. The ranging request includes the ranging parameters, e.g. the Ranging role (Reference UE or Target UE), one time or period ranging, ranging for distance or direction measurement or both.

Editor's note:	The parameters coordinated between the 2 UEs will be further evaluated.

UE1 can determinate the Ranging role based on the ranging authorization in step 1 or ranging capability (capability as Reference UE or Target UE). For example, the UE1 decides to act as Reference UE, then the Ranging role means that "I am Reference UE" or "you are Target UE".

UE1 can get the one time or period ranging, ranging for distance or direction measurement or both from the application layer when the application layer sends the ranging request to the upper layer (e.g. Ranging layer).

5.	UE2 sends the ranging response to the UE1. If UE2 wants to change the Ranging role (e.g. UE2 wants to act as Reference UE), for example due to its ranging capability, a new Ranging role is included.

6.	The upper layer of each UE provides the ranging configuration to the AS layer. The ranging configuration includes the Ranging role (Reference UE or Target UE), one time or period ranging, ranging for distance or direction measurement or both.

7.	The AS layer of each UE transmits or receives Ranging signalling according to the ranging configuration, and the Reference UE calculates the ranging results. For example, for direction measurement, Target UE transmits Ranging signalling and Reference UE receives it accordingly.

Editor's note:	Coordination with RAN WGs is needed on whether and how the ranging configuration is used.

8.	The ranging results could be shared between the UEs via e.g. the PC5-S signalling.

### 6.3.3	Impacts on services, entities, and interfaces

UE:

-	Receives the ranging authorization policy and parameters from PCF;

-	Inter-layer communication regarding the configuration for ranging operation;

-	Inter-UE communication regarding parameters negotiation for ranging service and possibly further sharing of the ranging results.

## 6.4	Solution #4: Ranging devices discovery and ranging procedure

### 6.4.1	Description

This solution is related to the Key Issue #3 on Ranging/Sidelink Positioning device discovery and Key issue #4: Control of Operations for Ranging/Sidelink positioning.

To realize the Ranging/Sidelink Positioning, as explained in KI#4, the signalling between the UEs is needed for the purpose of Ranging/Sidelink Positioning control to exchange the coordination & configuration information and the Ranging/Sidelink positioning signals measurement results, it, is very similar to the LPP procedure between UE and LMF over NAS (e.g. capability transfer, Assistance Data Transfer, Location Information Transfer specified in 6.4 of TS 38.305 [16]). Correspondingly, this solution is proposed to add a Ranging/Sidelink Positioning Protocol layer on top of existing direct communication protocol as specified in clause 6.1 of TS 23.304 [4], and clause 5.2.1 of TS 23.287 [3] for the Ranging/Sidelink Positioning control signalling interaction between UEs.

Figure 6.4.1-1: Ranging/sidelink Positioning protocol over PC5 reference point

Similar to LPP over Uu interface, the Ranging/Sidelink Positioning Protocol (RSPP) may include the following procedures as described in clause 6.4.2.4:

-	Procedures related to capability transfer for Ranging/Sidelink Positioning between UEs;

-	Procedures related to Assistance Data Transfer for Ranging/Sidelink Positioning between UEs;

-	Procedures related to Location Information Transfer for Ranging/Sidelink Positioning between UEs.

NOTE:	Details of the RSPP procedures in this solution are expected to be developed in RAN WGs.

Editor's note:	Texts related to RSPP may be further updated once there's any conclusion in RAN Working Groups.

Editor's note:	Whether a new Ranging/Sidelink positioning protocol is defined or an enhancement to LPP to support Ranging/Sidelink positioning is defined will be determined by RAN Working Groups.

### 6.4.2	Procedures of Ranging/Sidelink Positioning for 5G ProSe

#### 6.4.2.1	General

To realize the control of operations for Ranging/Sidelink positioning, following procedure is assumed to be performed:

-	Ranging/Sidelink Positioning device discovery;

-	Direct communication establishment for Ranging/Sidelink Positioning protocol procedure;

-	Ranging/Sidelink Positioning procedure.

The following procedures are for UEs that support 5G ProSe PC5 reference point as defined in TS 23.304 [4].

Editor's note:	Solutions on device discovery for UE supporting V2X PC5 reference as defined in TS 23.287 [3] may be provided in other solutions.

Existing Model A and Model B direct discovery as specified in clause 6.3.2.1 of TS 23.304 [4] are reused as basis for Ranging/Sidelink Positioning devices discovery.

Existing Unicast mode 5G ProSe Direct Communication establishment procedure as specified in clause 6.4.3 of TS 23.304 [4] are reused.

#### 6.4.2.2	Procedures of Ranging/Sidelink Positioning between 2 UEs with Model A discovery

Figure 6.4.2.2-1: High-level procedure of Ranging/Sidelink Positioning with Model A discovery

In this procedure, Reference UE and Target UE can take the design of UE Assisted and UE Based Positioning Procedure as specified in clause 6.11.1 of TS 23.273 [11] to enable the Ranging/Sidelink Positioning.

1.	The Target UE sends an Announcement message. The Announcement message may include the Type of Discovery Message, Target UE info, Ranging/Sidelink Positioning service Code, Role indication (target UE) and Target UE capability (e.g. ranging support).

2.	Reference UEs monitor the Announcement message. If the Target info is the Reference UE to be discovered, the Reference UEs have the interests on the Ranging service and support the Ranging/Sidelink Positioning capability of the Target UE if any, the Reference UE performs the direct PC5 link establishment procedure with target UE as described in clause 6.4.3.1 of TS 23.304 [4].

3.	The Target UE and the Reference UE perform the Ranging/Sidelink Positioning control interaction procedure to exchange the coordination & configuration information, e.g. the Ranging capability and Ranging Assistance Data, as described in clause 6.4.2.4.

NOTE:	The PC5 message carrying RSPP message is consider as new kind of PC5-S message in this solution.

Editor's note:	How to handle the definition of PC5-S message carrying RSPP message is FFS.

4.	Target UE and the Reference UE pass the interacted coordination & configuration information down to the AS layer.

5.	Target UE and the Reference UE perform the Ranging/Sidelink Positioning measurement and result calculation.

6.	The Target UE and the Reference UE perform the Ranging/Sidelink Positioning control interaction procedure to exchange Ranging/Sidelink Positioning measurement results as described in clause 6.4.2.5, and then provide the measurement results to the RSPP layer.

#### 6.4.2.3	Procedures of Ranging/Sidelink Positioning between 2 UEs with Model B discovery

Figure 6.4.2.3-1: High-level procedure of Ranging/Sidelink Positioning with Model B discovery

In this procedure, Reference UE and Target UE can take the design of UE Assisted and UE Based Positioning Procedure as specified in clause 6.11.1 of TS 23.273 [11] to enable the Ranging/Sidelink Positioning.

1.	The Reference UE sends a Solicitation message. The Solicitation message may include Type of Discovery Message, Ranging/Sidelink Positioning service Code, Reference UE info, Target UE info, Role indication (Reference UE) and Reference UE capability.

2.	Target UE monitors the Solicitation message. If the Target UE decides to be discovered and ranged based on the included Target info and the Reference UE capability in the Solicitation message, the Target UE responds to the Reference UE with a Discovery response message. The Discovery response message may include the Target UE capability.

3.	After the target UE discovery, the Reference UE performs the PC5 Unicast communication establishment with target UE.

4-7.	This step is same as the step 3~6 of Figure 6.4.2.2-1.

NOTE:	Reference UE can also initiate the discovery procedure to discover a Target UE.

#### 6.4.2.4	RSPP procedures to exchange the coordination & configuration information

##### 6.4.2.4.1	RSPP procedures to exchange Ranging capability

Figure 6.4.2.4.1-1: RSPP procedures to exchange Ranging capability

1.	For the RSPP procedure to exchange Ranging capability, the Target UE or the Reference UE may request the peer UE to provide the Ranging capability. The Ranging capability request is transferred via the PC5-S message.

2.	The Target UE or the Reference UE provides its Ranging capability to the peer UE which is transferred via the PC5-S message.

##### 6.4.2.4.2	RSPP procedures to exchange Ranging assistant data

Figure 6.4.2.4.2-1: RSPP procedures to exchange Ranging assistant data

1.	For the RSPP procedure to exchange Ranging assistant data, the Target UE or the Reference UE may request the peer UE to provide the Ranging assistant data. The Ranging assistant data request is transferred via the PC5-S message.

2.	The Target UE or the Reference UE provides its Ranging assistant data to the peer UE which is transferred via the PC5-S message.

#### 6.4.2.5	RSPP procedures to exchange Ranging/Sidelink Positioning measurement results

Figure 6.4.2.5-1: RSPP procedures to exchange Ranging/Sidelink Positioning measurement results

1.	For the RSPP procedure to exchange Ranging/Sidelink Positioning measurement results, the Target UE or the Reference UE may request the peer UE to provide the Ranging/Sidelink Positioning measurement results. The Ranging/Sidelink Positioning measurement result request is transferred via the PC5-S message.

2.	The Target UE or the Reference UE provides its Ranging/Sidelink Positioning measurement results to the peer UE which is transferred via the PC5-S message.

### 6.4.3	Impacts on Existing Nodes and Functionality

The solution has impacts on the following entities:

UE(s):

-	Support the RSPP over PC5 reference point.

-	Support the PC5-S message carrying RSPP message.

## 6.5	Solution #5: Solution to support Ranging/Sidelink Positioning

### 6.5.1	General

This solution addresses the KI#3: Ranging/ Sidelink Positioning device discovery and the KI#4: Control of Operations for Ranging/ Sidelink.

### 6.5.2	Functional Descriptions

To trigger the sidelink positioning procedure between an UE-1 and UE-2(s), the application layer in UE-1 provides ranging/sidelink positioning parameters to ranging layer. The ranging layer in the UE-1 broadcasts a ranging/sidelink positioning request message which includes the parameters provided by the application layer.

After the UE-2(s) receives the message, the sidelink positioning procedure is triggerred as follows to get the Ranging/Sidelink positioning result:

-	UE-1 based sidelink positioning: the UE-1 triggers the sidelink positioning procedure to obtain measurements and calculates the Ranging/Sidelink positioning result. Or

-	UE-1 assisted sidelink positioning: the UE-2(s) triggers the sidelink positioning procedure to obtain measurements, calculates the Ranging/Sidelink positioning result and returns it to the UE-1.

NOTE 1:	The details of the sidelink positioning procedure is in RAN WG scope.

NOTE 2:	Both the UE-1 and UE-2 can act as the reference UE. The capability exchange and reference UE negotiation between the UE-1 and UE-2 is in RAN WG scope. When the UE-1 obtains the Ranging/Sidelink positioning result, the ranging layer of the UE-1 provides it to the application layer.

### 6.5.3	Ranging/ Sidelink Positioning Procedures

Figure 6.5.3.1-1: Ranging/Sidelink Positioning Procedure

1.	Application layer provides sidelink positioning parameters to ranging layer, e.g. required location QoS, target UE's Application Layer ID may also be included in the parameters.

2.	UE-1 broadcasts a Ranging/Sidelink positioning request. The message includes UE-1's Application Layer ID, Target UE's Application ID and required QoS.

3.	UE-2 returns the Ranging/Sidelink Positioning response to UE-1 if the Application Layer ID for UE-2 matches.

4.	UE-1 initiates the Sidelink Positioning procedure. The details of the procedure are in scope of RAN WGs.

NOTE 1:	UE role (e.g. reference UE) configuration and authorisation is realized by the solutions for KI#1.

NOTE 2:	The details of step 4 are in RAN WGs. To better explain the solution, here lists some examples may be included in step 4, e.g. capability exchange, reference UE negotiation, Ranging/Sidelink positioning measurement and calculation, Ranging/Sidelink positioning result transmission from UE-2 to UE-1 if the UE-2 is responsible for calculation.

5.	The ranging layer in UE-1 provides the Ranging/Sidelink positioning result to the application layer.

### 6.5.4	Impacts on services, entities, and interfaces

PC5 interface is enhanced to support ranging/sidelink positioning.

## 6.6	Solution #6: Network assisted Ranging and Sidelink Positioning

### 6.6.1	Description

This solution is related to the Key Issue #5 on Network assisted Sidelink Positioning for In Network Coverage and Partial Network Coverage.

In this solution, the Located UE, who has capabilities that offers Sidelink Positioning and/or Uu based positioning, may broadcast its Network assisted Ranging and Sidelink Positioning service. A target UE who is not able to perform Uu based positioning estimation may call this Network assisted Ranging and Sidelink Positioning service to obtain its own Uu positioning estimation. Network assisted Ranging and Sidelink Positioning may require that the network has the capability of computing, providing the UE position or providing the assistant data to the UE.

The Network assisted Ranging and Sidelink Positioning can be realized based on the following potential mechanism:

-	Located UE requested procedure on behalf of the Target UE;

-	Target UE requested procedure.

For the Located UE requested procedure and Target UE requested procedure, the Network assisted Ranging and Sidelink Positioning may require that the network can compute and provide the UE position based on the Uu position of Located UE and the ranging information over sidelink. The ranging information over sidelink may be provided by the the Target UE or Located UE to the network.

### 6.6.2	Procedures of Located UE discovery

Figure 6.6.2.-1: Located UE discovery and selection

In this solution, a Located UE, who has capabilities that offers Sidelink Positioning and/or Uu based positioning, may broadcast its Network assisted Ranging and Sidelink Positioning service taking the discovery procedure as basis in clause 6.3.2.2 of TS 23.304 [4]. A UE who is not able to perform Uu based positioning estimation may call this Network assisted Ranging and Sidelink Positioning service to obtain its own Uu positioning estimation.

1.	The Located UE broadcasts the discovery procedure including a NW assistant ranging/sidelink positioning container. The NW assistant ranging/sidelink positioning container can reflect that the Located UE can provide the Network assisted Ranging and Sidelink Positioning service. The container may include the Located UE ranging/sidelink capability, Uu positioning capability. The Located UE may be aware of that its network supports the Network assisted Ranging and Sidelink Positioning based on the network indication during the registration procedure.

2.	Proximate Target UEs, who is not able to perform Uu based positioning estimation, monitors above discovery message and perform the Located UE selection based on above container (e.g. Located UE capability). After selection, the Target UE invoke the Network assisted Ranging and Sidelink Positioning service from the selected Located UE, as described in clause 6.6.3.

Editor's note:	How to support realize the Located UE discovery over V2X is FFS, e.g. carry discovery message as V2X application traffic.

### 6.6.3	Network assisted Ranging and Sidelink Positioning

#### 6.6.3.1	Located UE requested procedure on behalf of the Target UE

Figure 6.6.3.1-1: Located UE requested procedure with NW assistant ranging/sidelink positioning

1.	After the Located UE selection, the Target UE calls the NW assistant ranging/sidelink positioning service from the Located UE via the PC5 message.

2.	After receiving the above direct communication request, the Located UE may perform the Ranging/Sildelink positioning procedure with Target UE.

3.	The Located UE takes the 5GC-MO-LR Procedure as basis to perform the subsequent NW assistant ranging/sidelink positioning procedure. The Located UE sends a UL NAS TRANSPORT message with including a MO-NW assistant ranging/sidelink positioning request, the request may include the Ranging/Sildelink positioning assistant data based on the step 2.

4.	AMF may perform the LMF selection with NW assistant ranging/sidelink positioning condieration.

5.	AMF send a Nlmf_location_DetermineSidelinkLocation request to the selected LMF, the request includes the Ranging/Sildelink positioning assistant data.

6.	The LMF initiates the Located UE positioning as described in TS 23.273 [11].

7.	The LMF determine the Target UE position based on Ranging/Sildelink positioning assistant data and the Located UE position.

8.	The LMF responds to AMF with a Nlmf_location_DetermineSidelinkLocation response including the Target UE position.

9.	AMF sends a DL NAS TRANSPORT message to the Located UE, which include a MO-NW assistant ranging/sidelink positioning response with Target UE position.

Editor's note:	It is FFS how the Target UE privacy is handled when the Target UE location is disclosed to the Located UE.

10.	Located UE responds to Target UE with NW assistant ranging/sidelink positioning response including Target UE position.

#### 6.6.3.2	Target UE requested procedure

Figure 6.6.3.2-1: Target UE requested procedure with NW assistant ranging/sidelink positioning

1.	After a Located UE is selected , the Target UE performs the Ranging/sidelink positioning with Located UE.

2.	Target UE initiates MO-LR procedure, and sends a UL NAS TRANSPORT message with including a NW assisted ranging/sidelink MO-LR request, the request may include the Ranging/Sildelink positioning assistant data based on the step1. In addition, the Located UE ID (e.g. 5G GUTI) may be carried along withthe UL NAS TRANSPORT message.

3.	AMF performs the LMF selection with NW assistant ranging/sidelink positioning consideration. If the Located UE ID repents a different AMF, the AMF forwards the NW assisted ranging/sidelink MO-LR request along with Located UE ID to the AMF corresponding to the Located UE ID.

4.	AMF sends a Nlmf_location_DetermineSidelinkLocation request to the selected LMF, the request includes the Ranging/Sildelink positioning assistant data and the Located UE ID (e.g. GUTI).

5.	The LMF initiates the Located UE positioning as described in TS 23.273 [11].

6.	The LMF determine the Target UE position based on Ranging/Sildelink positioning assistant data and the Located UE position.

7.	The LMF responds to AMF with a Nlmf_location_DetermineSidelinkLocation response including the Target UE position. For different AMFs case, the AMF further forwards the response to the AMF of Target UE.

8.	AMF sends a DL NAS TRANSPORT message to the Target UE, which include a NW assisted ranging/sidelink MO-LR response with Target UE position.

NOTE:	In this procedure, it assumes the Target UE is able to establish NAS connection with AMF when the target UE is out-of-coverage, for example, via ProSe L2 NW relay, ProSe L3 NW relay with N3IWF, Non-3gpp access connected to 5GC. If the Target UE is in coverage, the NW assisted ranging/sidelink MO-LR procedure can be performed with Target UE directly.

### 6.6.4	Impacts on Existing Nodes and Functionality

The solution has impacts on the following entities:

AMF:

-	support the LMF selection with NW assistant ranging/sidelink positioning.

-	is able to support the NW assistant ranging/sidelink positioning request from the Target UE and Located UE.

LMF:

-	Support the UE position determination based on the Uu position and sidelink position.

UE(s):

-	Support the NW assistant ranging/sidelink positioning request.

## 6.7	Solution #7: Network based UE Sidelink Positioning

### 6.7.1	Introduction

This solution relates the KI#5 and proposes a solution where the network perform the UE positioning estimation based on network assisted Sidelink positioning.

### 6.7.2	Functional Description

This solution addresses KI#5 for MT-LR case, and the following principles are used:

-	UE within the network coverage, either directly over Uu or via a L2 U2N relay, is in a location where Uu based positioning is not possible

-	MT-LR procedure is triggered by the network as specified in TS 23.273 [11] clause 6.1.

-	UE is requested to perform positioning measurements by the LMF

-	The UE check whether the UE can perform positioning measurements on other RAN nodes or needs to use Sidelink positioning methods. The UE determines that Uu based positioning is not possible and needs to perform Sidelink positioning. The UE search for another UE that is able to estimate it's own location based on Uu measurements and support Ranging service.

NOTE 1:	The discovery of the Located UE depends on solution for KI#3 "Ranging/Sidelink Positioning device discovery"

-	The UE performs ranging estimate measurements of the Located UE and provides either the ranging estimate or the ranging measurements data to the LMF. The UE also include the UE ID of the Located UE.

NOTE 2:	LPP (TS 37.355 [10]) may need to be enhanced to support new measurement message to support Sidelink positioning.

-	When the network received the information from the UE, the network can use a known location of the Located UE or trigger a new location estimate from the Located UE. The LMF triggers any of the location procedures specified in TS 23.273 [11] clause 6.11 to estimate the Located UE's position.

-	Based on the position estimate of the Located UE, the LMF can estimate the UE position, the UE being the UE that the first positioning request was sent to.

NOTE 3:	The RAN study on Sidelink Positioning will determine the accuracy of Sidelink position e.g. whether multilateration positioning estimate(s) are needed or not.

### 6.7.3	Procedures

#### 6.7.3.1	Network based UE Sidelink Positioning estimation (MT-LR).

Figure 6.7.3.1-1: Network based UE Sidelink Positioning estimation (MT-LR)

0.	The AMF is triggered to initiate location reporting procedure for UE1 as specified in TS 23.273 [11]. UE1 may either be in network coverage and communicates over Uu, or UE1 uses a Layer-2 UE-to-Network Relay to communicate with the network. The Layer-2 UE-to-Network Relay can be the Located UE or a different UE that can offer Layer-2 UE-to-Network service.

NOTE 1:	UE1 and UE2 may be registered and served by different AMFs.

1.	The AMF sends the initial request to the LMF as specified in TS 23.273 [11]

2.	The LMF sends the DL Positioning message to the UE1 using the Namf_Communication_N1N2messageTransfer as specified in clause 6.11.1 of TS 23.273 [11].

3.	The UE1 decides to perform discovery procedure to find a UE that supports sidelink positioning and Uu based measurements (e.g. because it has determined it is unable to perform legacy Uu positioning measurements). The UE1 selects UE2.

NOTE 2:	The UE logic to determine to use sidelink positioning is based on UE implementation. E.g. the UE could determine that Uu positioning is not possible for instance based on the number of cells that UE1 can detect being limited.

NOTE 3:	It is assumed that solutions for discovery addressing the KI#3 can used. During the discovery procedure UE1 will get the UE_ID of UE2 which supports positioning procedures defined in TS 23.273 [11]. The UE_ID is the ID that UE2 used during the ProSe discovery procedure.

4.	The UE1 perform Ranging/Sidelink relative distance measurements to the UE2.

NOTE 4:	It is assumed that solutions addressing KI#4 can used.

5.	The UE1 send the UL Positioning message to the LMF. This message includes both the Ranging/Sidelink positioning data and the UE_ID of UE2.

6.	The LMF resolves the received UE_ID of UE2 by querying the DDNMF and triggers the location determination of UE2. The LMF may need to query the UDM to find the AMF that is serving the UE2 to initiate the positioning procedures.

NOTE 4:	It is assumed that the LMF uses the service based interface N5g-ddnmf for interaction with the DDNMF.

7.	The LMF triggers the serving AMF to initiate the positioning procedure. The LMF receives from the AMF the LCS Correlation ID and cell-ID for UE2 and performs one or more of the positioning procedures described in clauses 6.11.1, 6.11.2 and 6.11.3 of TS 23.273 [11] to determine the location of UE2. The LMF determines the location of UE1 based on the SL positioning data received in step 5 and the location of UE2.

NOTE 5:	UE1 mobility may impact the LMF location estimation of UE2. To reduce the impact the LMF may try to time synchronize the UE1 location measurements and the SL measurements or use time stamps and estimate the mobility based on mobility trajectory. This is a generic topic for all solution that use non-stationary devices that participates in the location estimation of another UE.

8.	The LMF provides the UE1 location to the AMF as a response to the request in step 1.

### 6.7.4	Impacts on services, entities and interfaces

UE:

-	Support Ranging and Sidelink positioning.

LMF:

-	Support the Ranging and Sidelink positioning data and ability to calculate the location of a UE based on this data set and the location of the Located UE (UE2).

-	Resolve the UE_ID of the Located UE (UE2). The UE_ID being received in the UL positioning message together with the SL position data.

## 6.8	Solution #8: Discovery of a Located UE for Enhancing Uu based Positioning in Network Coverage

### 6.8.1	Introduction

This solution relates to discovery of a Located UE (UE2) with ranging/sidelink positioning capability which can potentially be exploited as additional reference nodes to enhance Uu based positioning of the first UE (UE1) under insufficient network coverage.

The following assumptions can be made for UE1 and UE2:

UE1:	A UE whose location is to be determined using Uu based positioning. The positioning capability of UE1 on Uu is defined in TS 37.355 [10]. UE1 is capable of ranging/sidelink positioning to another UE over PC5. UE1 is considered to be in coverage.

UE2:	A UE whose location is available to some degree of accuracy, which is to be exploited for assisting positioning of the other UE, e.g. UE1. It is assumed that UE2 supports location service over Uu and ranging-based service/sidelink positioning over PC5. UE2 is considered to be in coverage.

In the terminology of ranging-based services and sidelink positioning, UE1 and UE2 can be both a Target UE and a Reference UE.

### 6.8.2	Functional Description

In order to discover the direct links between UE1 and UE2 that may assist positioning of UE1, the 5G ProSe Direct Discovery is to be triggered by Location Management Function (LMF) based on LCS requirements. The LMF sends the Application ID received from the AF to the UE to trigger the UE preforming the 5G ProSe Direct Discovery. LMF may also determine the discovery model for both UE1 and UE2.

When the location request is from the external AF, the GMLC will map the external Application ID provided by the AF to a dedicated Application ID. The external Application ID is an Application ID defined in the TS 23.303 [22] to indicate which application want to know the UE location. The internal Application ID has the same function as the Prose Application ID defined in the TS 23.303 [22] to trigger the UE to preform Prose direct discovery. Maybe there is only one internal Application ID because such ID is just used to trigger the discovery and do not need to distinguish which application sends the request.

Given a number of UEs like UE2 within the network coverage with known location and ranging/sidelink positioning capability, a subset of them may be triggered to initiate direct discovery over PC5. Then, they interact with 5GC to obtain 5G ProSe service authorization and the corresponding ProSe Direct Discovery parameters.

The Discovery messages is to be used by UE2 to indicate its capability in terms of assisting Uu based positioning as well as ranging/sidelink positioning. The content of Discovery message is to be provided by LMF and may consists of the following:

-	UE identity (UE2).

-	Capability information of UE2, including positioning capability over Uu and ranging/sidelink positioning capability over PC5. The latter, for example, may include capability of transmitting and/or processing sidelink positioning signals.

-	Location information.

Correspondingly, LMF triggers UE1 with ranging/sidelink positioning capability to monitor the Discovery message on PC5.

Alternatively, LMF may configure UE1 to transmit positioning assistance requests. In this case, the Discovery message may consists of:

-	UE identity (UE1).

-	Requested positioning capability over Uu and ranging/sidelink positioning capability over PC5. The latter, for example, may include capability of transmitting and/or processing sidelink positioning signals.

-	Requested location information, e.g. accuracy.

Correspondingly, LMF triggers UE2 with ranging/sidelink positioning capability to respond to the Discovery message with related information on PC5.

### 6.8.3	Procedures

This clause provides a signalling procedure for PC5 assisted Network based positioning.

Figure 6.8.3-1: Procedure for network-based positioning assisted by ranging/sidelink positioning

Signalling procedures for supporting Uu-based/network-based positioning with ranging/sidelink positioning shown in Figure 6.8.3-1 can be described as the following:

1.	The AF sends the location request for the target UE (UE1) directly to the GMLC or via the NEF including the Application ID to indicate which application wants to know the UE location. The LCS QoS may also be contained in the request.

2.	If the request is from the external AF, the GMLC maps the external Application ID to the internal Application ID.

3.	GMLC sends the location request to the LMF via AMF as described in TS 23.273 [11].

4.	Based on the request, the LMF may first trigger the Uu based positioning for UE1. LCS procedures for positioning UE1 are carried out according to TS 23.273 [11].

5.	Based on the requested LCS QoS, LMF determines to trigger discovery of a Located UE for positioning assistance.

6.	LMF obtains capability information in terms of LCS and ranging/sidelink positioning and available location information of a number of UEs within a region of interest, e.g. tracking area, geographical zone, and triggers particular UEs, e.g. UE2, to discover UE1 reusing 5G ProSe Direct Discovery procedure. LMF sends the Application ID to the UE2 to trigger the UE2 discovering UE1 reusing 5G ProSe Direct Discovery procedure. The Application ID can be the internal Application ID provided by the GMLC in step 2. LMF configures the information to be contained in the Discovery message, e.g. UE identity, indication of capability information and/or location information.

NOTE 1:	LMF is assumed to have obtained the capability/location information of UE2 either by inquiring PRU info or by carrying out Uu based location estimation before this procedure.

7.	LMF sends the Application ID to the UE1 to trigger UE1 discovering UE2 for positioning assistance reusing 5G ProSe Direct Discovery procedure. The Application ID can be the internal Application ID provided by the GMLC in step 2. LMF configures the information to be contained in the Discovery message. LMF may configure UE1 to send a discovery report to LMF and specify information to be included in the report, e.g. discovered UE identity(UE2).

8.	UE1 and UE2 perform 5G ProSe Direct Discovery procedure as defined in clause 6.3.2 of TS 23.304 [4].

If Model A discovery is used,UE2 broadcasts a Discovery message containing UE identity, indication of capability information and/or location information as configured by LMF in step 3.UE1 monitors Discovery message.

If Model B discovery is used, UE1 transmits a Discovery message containing UE identity, requested capability information and/or location information as configured by LMF. UE2 that receives the request message may respond to UE1.

9.	Based on the result of direct discovery, UE1 provides a discovery report to LMF as configured in step 4. The discovery report may contain the discovered UE with required capability and/or location information for positioning assistance in its proximity, e.g. UE2.

UE1 may send one or multiple discovery report messages, if it discovers more than one UE2.

10.	Based on the discovery report, the LMF determines which UE as a Located UE and in addition to the capability information and location information of the two UEs, LMF determines whether and how ranging/sidelink positioning between UE1 and UE2 should be carried out.

11.	LMF informs UE1 to perform ranging/sidelink positioning operation with UE2.

12.	UE1 and UE2 carry out ranging/sidelink positioning operations over PC5.

NOTE 2:	Step 9 is based on the Solution to KI#4: control of operations for Ranging/Sidelink positioning.

13.	UE1/UE2 provides ranging/sidelink positioning measurements to LMF.

NOTE 3:	Step 10 is for future study in FS_NR_pos_enh2 in RAN WGs (RP-213588).

14.	LMF determines and/or verifies the location estimate of UE1 based on the positioning measurements obtained over both Uu and PC5.

15.	LMF sends the location response to the GMLC including the UE1 location result via AMF.

16.	GMLC sends the location response to the AF including the UE1 location result.

Editor's note:	The current solution just support the UE1 and UE2 serving by the same LMF and for the case where the UE1 and UE2 have different serving AMF and LMF is FFS.

### 6.8.4	Impacts on services, entities, and interfaces

LMF:

-	Trigger UE discovery based on LCS QoS verification.

-	Configure content of the discovery message.

-	Determine whether and how a Located UE to be involved in ranging/sidelink positioning for assisting Uu-based positioning.

-	Determine LCS target UE's location based on location information obtained over Uu and PC5.

-	Send the Application ID to the UE.

UE:

-	Carry out positioning assisting UE discovery procedure based on LMF configuration.

GMLC:

-	Map the external Application ID to the internal Application ID.

AF

-	Send the Application ID to the GMLC directly or via NEF.

## 6.9	Solution #9: Solution for KI#6 on Ranging/SL positioning service exposure to UE

Editor's note:	This clause describes a solution addressing one or more key issues identified in clause 5. The structure of the (sub) clauses can be adjusted. The list of key issues which this solution attempts to resolve should be clearly indicated.

### 6.9.1	General

The solutions address the ranging and sidelink positioning service exposure to UE for Key Issue #6.

The followings give two potential cases and the corresponding solutions:

Case 1:	Two UEs, the UE involved in Ranging/SL positioning consumes the Ranging/SL positioning service.

Case 2:	Three UEs, the 3rd party UE consumes the Ranging/SL positioning service.

### 6.9.2	Functional descriptions

Ranging/SL positioning refers to the determination of the distance between two UEs or more UEs and/or the direction and/or relative positioning of one UE (i.e. Target UE) from another UE (i.e. Reference UE) via PC5 interface.

The consumer can be the UE involved in the Ranging/SL positioning service, or a 3rd party UE. The UE/3rd party UE may perform the Ranging/SL positioning operation directly via PC5 interface and use the result locally, without involvement of 5GC, or the 3rd party UE (e.g. out of SL coverage of either UE subject to Ranging/SL positioning) requests Ranging/Sidelink positioning service with the involvement of 5GC (e.g. AMF, LMF, etc.).

### 6.9.3	Procedures

#### 6.9.3.1	UE involved in SL ranging consumes the Ranging/SL positioning service

Figure 6.9.3.1-1: UE involved in SL ranging consumes the Ranging/SL positioning service

In this case, two UEs are involved in the Ranging/SL positioning operation. The UE triggers ranging service, performs the Ranging/SL positioning operation, and makes the calculation locally, i.e. Ranging/SL positioning service is exposed to one of the two UEs.

Figure 6.9.3.1-2: Procedure of ranging UE consumes the Ranging/SL positioning service

1.	The application layer of the UE1 triggers the request for Ranging/SL positioning.

2.	The UE1 initiates the Ranging/SL positioning operation with the UE2. The NG-RAN node is involved if UE is in network coverage and the scheduled resource allocation is used.

NOTE:	The role of Target UE and Reference UE is negotiated between UE1 and UE2 over PC5 interface in this step which can refer to the Solutions of KI#3&4.

Editor's note:	The security and privacy consideration for the Ranging/SL positioning operation should be studied by SA WG3.

3.	The UE1 calculates the result and uses it locally.

#### 6.9.3.2	3rd party UE consumes the Ranging/SL positioning service

##### 6.9.3.2.1	General

Figure 6.9.3.2.1-1: 3rd party UE consumes the Ranging/SL positioning service

In this case, three UEs are involved in the Ranging/SL positioning, in which a 3rd party UE asks for the Ranging/sSL positioning for the other two UEs (i.e. UE1 and UE2). The 3rd party UE may request via Uu interface to perform Ranging/SL positioning operation between UE1 and UE2 with the involvement of 5GC. If the 3rd party UE is in the sidelink coverage with one of the requested UEs (e.g. UE1), the 3rd party UE may alternatively request UE1 via PC5 interface to perform Ranging/SL positioning with UE2.

When 5GC is involved, there are network based Ranging/Sidelink positioning in which the Ranging/Sidelink positioning estimation result calculation is performed at LMF and UE based Ranging/Sidelink positioning in which the Ranging/Sidelink positioning estimation result calculation is performed at the UE.

##### 6.9.3.2.2	Service exposure to the 3rd party UE via PC5

Figure 6.9.3.2.2-1: Service exposure to the 3rd party UE via PC5

1.	The application layer of the 3rd party UE triggers the request for the Ranging/SL positioning, and the 3rd party UE requests the UE1 to perform the ranging/sidelink positioning operation with UE2.

2.	The UE1 initiates the Ranging/SL positioning operation with the UE2. The NG-RAN node is involved if UE is in network coverage and the scheduled resource allocation is used.

NOTE:	The role of Target UE and Reference UE is negotiated between UE1 and UE2 over PC5 interface in this step which can refer to the Solutions of KI#3&4.

Editor's note:	The security and privacy consideration for the Ranging/SL positioning operation should be studied by SA WG3.

3.	The UE1 makes the calculation locally.

4.	The UE2 returns the Ranging/SL positioning result to the 3rd party UE via the PC5 interface.

##### 6.9.3.2.3	Service exposure to the 3rd party UE via 5GC/Uu (network based)

Figure 6.9.3.2.3-1: Service exposure to the 3rd party UE via 5GC/Uu (network based)

1.	The application layer of the 3rd party UE triggers the Ranging/SL positioning, and then requests the network to perform the Ranging/SL positioning between the UE1 and UE2. If the 3rd party UE in CM-IDLE state, it firstly triggers the service request procedure.

2.	The 3rd party UE requests the network to perform the Ranging/SL positioning operation between the UE1 and UE2. The Ranging/SL positioning requirement may include the requirement regarding Ranging/SL positioning demand (i.e. the distance and/or the direction between two UEs or more UEs), delay, accuracy, result feedback mode (e.g. immediately, periodically or at a later time). The Ranging/SL positioning requirement is included in the MO-LR Request and sent to the LMF by the AMF. The 3rd party UE may determine the initiator UE (i.e. which is used to perform signalling exchange with the LMF) based on implementation and indicate which UE is the initiator UE to the AMF.

3.	The AMF3 check whether there is UE1 or UE2 context in the LMF3, if the AMF3 happens to have the context of UE1 or UE2, the AMF3 determines the initiator UE from UE1 and UE2. For example, if AMF3 only has UE2 context, then UE2 can be selected as the initiator UE. The AMF3 can also take the indication from the 3rd party UE, UE1 and UE2 capability, UE1 and UE2 current status (e.g. reachable or unreachable) by querying the UDM and operator policy into consideration when selecting the initiator UE. An unreachable UE would not be selected. If the AMF3 has UE2 context as an example, it selects an appropriated LMF (e.g. AMF3) for the UE2 and performs Option A; or else, the Option B is performed.

Option A:

4.	If the 3rd party UE is authorized to request the Ranging/SL positioning information between the UE1 and UE2 based on the subscription of 3rd party UE, and the AMF3 has UE2 context, it sends the Nlmf_Location_DetermineLocation request (UE1 ID, UE2 ID) to the LMF3 for the Ranging/SL positioning between theUE1 and UE2.

5a.	The LMF3 triggers the UE2 to perform Ranging/SL positioning with the UE1 via UE2's AMF (i.e. AMF3). The measurement request message may include the Ranging/SL positioning demand, UE1 ID, UE2 ID, etc.

5b.	The UE2 performs the Ranging/SL positioning operation with the UE1. The NG-RAN node is involved if UE is in network coverage and the scheduled resource allocation is used.

NOTE:	The role of Target UE and Reference UE is negotiated between UE1 and UE2 over PC5 interface in this step which can refer to the Solutions of KI#3&4.

Editor's note:	The security and privacy consideration for the Ranging/SL positioning operation should be studied by SA WG3.

5c.	The UE2 reports the Ranging/SL positioning measurements to the LMF for calculation.

5d.	The LMF3 makes calculation based on the Ranging/SL positioning measurements.

6.	The LMF3 returns the Ranging/SL positioning result to the AMF3.

Option B:

7.	If the AMF3 has neither context of UE1 and UE2, it may select the initiator (e.g. UE2) randomly or based on operator implementation and acts as an LCS client to request the GMLC for Ranging/SL positioning result. The GMLC triggers the Ranging/SL positioning procedure toward UE2's LMF2 via its AMF2 and gets the result. Then the GMLC returns the result to the AMF3.

8.	The AMF3 returns the result to the 3rd party UE.

### 6.9.4	Impacts on services, entities, and interfaces

UE:

-	sends Ranging/SL positioning requirement to another UE for Ranging/SL positioning measurement or to the LMF for Ranging/SL positioning result.

-	indicate which UE is the initiator UE to the LMF.

-	receives Ranging/SL positioning measurement from another UE or Ranging/SL positioning result from the LMF (via the AMF).

-	receives Ranging/SL positioning measurement request from the LMF, performs Ranging/SL positioning measurement, and reports the Ranging/SL positioning measurement to the LMF.

NG-RAN:

-	allocates PC5 radio resource to UE for Ranging/SL positioning operation.

AMF:

-	selects the initiator UE from UE1 and UE2.

-	acts as the consumer of GMLC service.

-	selects LMF and routes the Ranging/SL positioning requirement to the LMF.

-	routes the signalling (e.g. LPP messages for SL ranging) between the UE and the LMF.

-	Support check the 3rd party UE is authorized to request the Ranging/SL positioning information between the UE1 and UE2.

LMF:

-	requests and controls the UE to perform Ranging/SL positioning measurement.

-	receives the Ranging/SL positioning measurement from the UE and makes calculation.

-	returns the Ranging/SL positioning result to the UE via AMF.

## 6.10	Solution #10: Ranging service exposure to UE

### 6.10.1	Description

This solution is related to the Key Issue #2: Ranging service operation procedure with the assistance of another UE and the Key Issue #6 on Ranging and sidelink positioning service exposure to a UE.

In this solution, a SL Positioning Client UE consumes the Ranging/Sidelink positioning service from an Assistant UE. The Assistant UE uses the direct discovery procedure as specified in clause 6.3.2.1 of TS 23.304 [4] to expose its Ranging/Sidelink positioning service for the SL Positioning Client UE. The SL Positioning Client UE is not required to be Ranging/Sidelink positioning capable. As described in Figure 6.10.1.1-1, the SL Positioning Client UE performs the Assistant UE discovery and selection based on the Ranging information in the discovery message. The Ranging information may include the role indication (i.e. "Assistant UE"), Assistant UE's ranging capability, the level of ranging accuracy, ranging method and info of the list of UE that the Assistant UE can range with. After that, the SL Positioning Client UE invokes the Ranging/Sidelink positioning service provided to the selected Assistant UE, requesting Ranging result between UE1 and UE2. The Assistant UE performs the Ranging procedure with UE1 and with UE2 respectively based on the solutions of KI#4, and calculates the ranging result between UE1 and UE2 based on the ranging result between UE1 and itself and the ranging result between UE2 and itself. In this solution, the SL Positioning Client UE may be one of UE1 and UE2.

Figure 6.10.1.1-1: High-level procedure for ranging service exposure via Assistant UE

### 6.10.2	Procedures of Ranging service exposure to SL Positioning Client UE

#### 6.10.2.1	Procedures of Ranging service exposure to SL Positioning Client UE

Figure 6.10.2.1-1: High-level procedure of Ranging/Sidelink Positioning with Model A discovery

1.	The Assistant UE performs the model A discovery as specified in clause 6.3.2.1 of TS 23.304 [4], the announcement message may include the Assistant UE info, role indication (i.e. "Assistant UE"), ranging service code and Ranging exposure info. Ranging exposure info may include the list of UEs info that the Assistant UE can perform the Ranging and Sidelink positioning with, and the level of the ranging accuracy.

2.	If a SL Positioning Client UE has the requirement to invoke the Assistant UE's ranging service, based on the Ranging exposure info, the SL Positioning Client UE performs the Assistant UE discovery and selection. For the selected Assistant UE, the SL Positioning Client UE may perform the direct link establishment procedure with Assistant UE.

3.	To invoke the Assistant UE's ranging service, the SL Positioning Client UE sends a ranging service request to Assistant UE via the PC5 message, requesting Ranging estimation of UE1 and UE2. In addition, the PC5 message may include the SL Positioning Client UE info, Assistant UE info.

4.	After receiving the Ranging service request from the SL Positioning Client UE, the Assistant UE may perform the authentication procedure to determine whether the SL Positioning Client UE is authorised to invoke this Ranging service (e.g. to obtain the ranging result between UE1 and UE2).

Editor's note:	It is FFS on authentication procedure.

5.	If the SL Positioning Client UE is authorised, the Assistant UE performs the ranging and positioning procedure with the target UE(s) based on the solutions of KI#4.

6.	The Assistant UE obtains the ranging measurement of each target UE and calculates the Ranging result between the Target UEs.

7.	After that, Assistant UE responses to the SL Positioning Client UE with the Ranging service response. The ranging service response includes the ranging result between target UEs.

NOTE:	In this solution, the SL Positioning Client UE can be one of UE1 and UE2.

#### 6.10.2.2	Deferred procedures of Ranging service exposure to the SL Positioning Client UE

Figure 6.10.2.2-1: Deferred procedures of Ranging service exposure to the SL Positioning Client UE

1~7.	The step1~5 are similar to the step1~5 of the clause 6.10.2.1. Difference is that the Ranging service request from the SL Positioning Client UE may include the deferred event. The event may be as follows:

-	Periodic ranging: An event where a defined periodic timer expires, then activates a ranging procedure and ranging result report.

Editor's note:	what are the other events that may trigger Ranging/Sidelink positioning service is FFS.

8.	After the step 6, the Assistant UE detects the event.

9.	When the event is detected, the Assistant performs the Ranging/Sidelink positioning procedure with UE1/UE1 based on the solution of KI#4.

10.	Assistant UE re-calculate the ranging result and reports the ranging result between UE1 and UE2 included in the ranging service response to the SL Positioning Client UE via the PC5 message.

### 6.10.3	Impacts on Existing Nodes and Functionality

The solution has impacts on the following entities:

UE(s):

-	Support ranging service exposure.

-	support of invoking the ranging service.

-	support Ranging/Sidelink positioning over the PC5.

## 6.11	Solution #11: UE using exposure API

### 6.11.1	General

This solution relates the KI#6 and proposes a solution where a UE can use exposure API over user plane to access the Ranging and sidelink positioning service exposure.

### 6.11.2	Functional Description

This solution addresses KI#6 and the following principles are used:

-	UE register and include in the registration request that it would like to use the service API for Ranging and Sidelink positioning.

-	The AMF validate with the UDM that the UE is authorized to use the service API

-	The AMF provides to the UE the "Onboarding Enrolment Information" in the Registration Accept message

-	When the UE decides to use the service API it follows the procedures specified in TS 23.222 [12].

-	The UE start by performing the Onboarding of an API Invoker procedure in clause 8.1 ofTS.23.222 [12] and uses the received "Onboarding Enrolment Information" in the security procedures as specified in clause 6.1 of TS 33.122 [13].

-	Once Onboarded the UE performs the procedure for "API invoker obtaining authorization to access service API". The Service API here is the Ranging and SL positioning service exposure API, which is assumed to be specified for Key Issue #7.

-	The UE can now use the Service API as specified for Ranging and SL positioning service exposure.

NOTE:	Key Issue 7 address the Service API to be used by Application Functions and this solution reuses this Service API and it therefore not further detailed.

### 6.11.3	Procedures

#### 6.11.3.1	UE access to Ranging and sidelink positioning service exposure.

Figure 6.11.3.1-1: UE access to Ranging and sidelink positioning service exposure

1.	The UE send a Registration Request that includes an indication that the UE would like access to the Ranging and sidelink positioning service exposure provided via the NEF. This indication could be included within the UE and Network capability exchange or explicitly signalled.

2.	The AMF obtains the Ranging and sidelink positioning subscription data by invoking the Nudm_SDM_Get.

NOTE 1:	It is assumed that the authorization information to use the Ranging and sidelink positioning service is stored in the UE's subscription data.

3.	The UDM provides the Ranging and sidelink positioning subscription data to the AMF. The AMF verifies that the UE is authorized to use the service API. The Ranging and sidelink positioning subscription data also include the information that the UE needs for the initial Onboarding process as described in TS 23.222 [12] and TS 33.122 [13].

NOTE 2:	More NF e.g. UDR, AUSF may be involved in this step. SA3 will need to specify how and where the information for onboarding is generate and stored. In TS 33.122 [13] the information for onboarding is called Onboarding Enrolment Information.

4.	The AMF includes the information for onboarding in the Registration Accept message to the UE.

5.	The UE initiates the Onboarding procedure as specified in TS 23.222 [12] clause 8.1 over an PDU session that provide access to data network that is connected to the NEF.

6.	The UE sends an obtain service API authorization request to the NEF and based on the UE's subscription information the authorization information to access the service APIs is sent to the UE in the response. See clause 8.11 of TS 23.222 [12].

7.	The UE uses the Ranging and sidelink positioning service exposure API to gain access to Ranging and Sidelink information related to other UEs.

NOTE 3:	The UE solution reuses this Service API proposed in solutions for Key Issue 7 address used by Application Functions.

### 6.11.4	Impacts on services, entities and interfaces

UE:

-	Support Ranging and Sidelink capability exchange.

-	Support receiving the Onboarding enrolment information, to be used in the initial access to the NEF.

AMF:

-	Support the Ranging and Sidelink capability exchange and to retrieve the Onboarding enrolment information from the NF that holds this information e.g. the UDM. Provide the Onboarding enrolment information to the UE in the Registration Accept message.

UDM:

-	Store the Onboarding enrolment information and provide it to the AMF when requested.

## 6.12	Solution #12: LCS-based Ranging Service Exposure

### 6.12.1	General

This solution addresses KI#7 as described in clause 5.7 and KI#5 in clause 5.5 by proposing a ranging service exposure solution in the 5GS towards the AF that leverages the existing ProSe-based UE-to-Network Relaying mechanism and the LCS architecture. Therefore, the procedure can be used both to determine relative positioning between UEs and absolute locations of UEs via ranging and sidelink positioning operations.

### 6.12.2	Functional descriptions

At a high level, the functional description of this solution can be summarized as follows:

-	A ranging exposure service is proposed enabling a ranging application server hosted at an AF to send a request to the 5GS to trigger a ranging/sidelink positioning operation between two UEs, one of which acts as reference UE the other ones as target UEs. The request may also indicate if the absolute location of the target UE(s) is desired.

-	The proposed solution leverages the LCS architecture for localization, i.e. GMLC and LMF are both involved in the 5GS procedure triggering the ranging/sidelink positioning given the close relation of ranging and sidelink positioning for a UE with the determination of the UE location. One UE (i.e. a located UE acting also as reference UE) acts as a ProSe UE-to-Network Relay for location reporting.

-	LMF is the entity in charge of performing the interactions with the ProSe Application Server and the PCF.

-	The located UE acting also as reference UE is configured or provisioned with special Relay Service Codes (RSCs) authorized for reporting of target UE location when acting as a relay.

-	The positioning of the set of UEs not configured as relays (i.e. target UEs) can be performed by the reference UE via sidelink positioning in RAN. This set of UEs are authorized to act as relayed UE for location reporting.

-	The service operation may include a number of ranging/sidelink positioning events related to both distance and direction measurements.

NOTE:	An assumption for this solution is that target UEs cannot be located via Uu positioning when an absolute location is requested. In that case, sidelink positioning is applied.

### 6.12.3	Procedures

Figure 6.12.3-1: Procedure for LCS-based ranging service exposure

Figure 6.12.3-1 shows a procedure with the proposed service exposure framework towards the application server. The description of the procedure is as follows:

0.	The ProSe capabilities of the UEs are registered with the AMF beforehand. This may include the support for location reporting in addition to other ProSe capabilities. Also the Location Privacy Indication (LPI) defined in TR 38.845 [6] for ProSe based localization can be given by the UE's at this stage, again to the AMF.

1.	Ranging/sidelink positioning service request (via NEF): the ranging application server may contact GMLC directly or via NEF to provide a ranging or sidelink positioning operation request to the 5GS. The request may be for one or multiple target UEs, and may indicate if the server requires relative positioning of target UEs, absolute positioning of target UEs, or both.

The service operations involved in the ranging/sidelink positioning request may reuse and extend the existing NEF/GMLC services (e.g. Nnef_EventExposure_Subscribe, Ngmlc_Location_ProvideLocation) and/or new service operations may be defined if needed. The service operations may include the target and reference UEs, the required Ranging/Sidelink positioning QoS, and a number of events addressing distance and/or direction measurements between the reference and target UEs. The service operations may also indicate threshold(s) for distance and/or direction events. In addition, the service operations may include other event configuration parameter(s) that may be mandatory or optional (e.g. time to trigger, direction orientation, threshold matching direction, measurement frequency, preferred accuracy, privacy preservation).

2.	Setup of location information as per 5GC-MT-LR procedure in TS 23.273 [11], corresponding to steps 2-11 in Figure 6.1.2-1 contained in clause 6.1.2, potentially including new optional inputs as part of the existing service operations accounting for the ranging related events.

The following enhancements of the 5GC-MT-LR procedure in TS 23.273 [11] are required to support ranging/sidelink positioning:

-	Step 5: an AMF service is consumed by GMLC to request ranging/sidelink positioning. The service operation can be Namf_Location_ProvidePositioningInfo with the same enhancements described above for Ngmlc_Location_ProvideLocation or a new service operation could be defined if needed.

-	Step 10: an LMF must be selected that supports ranging/sidelink positioning.

-	Step 11: either the Nlmf_Location service operations should be extended to support ranging/sidelink positioning with the enhancements described for Ngmlc_Location_ProvideLocation service operation or a new service operation should be defined for that purpose.

3.	If not done yet, the LMF checks the ProSe capabilities registered for the UEs in step 0 with the AMF.

4.	For the UEs supporting ProSe capabilities including also location reporting, the LMF interacts with the ProSe application server (or PCF) to initiate Relay operation for location reporting.

5.	[OPTIONAL] If not done previously, the ProSe AS may authorize and/or configure UE1 as the reference UE through which the other UEs (similar to ProSe Remote UEs) can relay location information to LMF.

The reference UE (acting similar to a ProSe UE-to-Network Relay as specified in TS 23.304 [4]) may be configured or provisioned with special Relay Service Codes (RSCs) authorized for location reporting. This is provided by the PCF, already provisioned in the ME or configured in the UICC.

UE1 in Figure 6.12.3-1 is configured as ProSe L2 NW relay

Editor's note:	Whether UE1 in Figure 6.12.3-1 can be configured as ProSe L3 NW relay is FFS.

6.	If not done previously, the ProSe application server authorizes the target UEs. It also configures or provisions target UEs with special RSCs authorized for location reporting. This is provided by the PCF, already provisioned in the ME or configured in the UICC.

7-8.	The ProSe Relay Discovery on the UEs identified in Steps 3-5 is conducted with special RSCs based on either model A or Model B as specified in cl. 6.3.2.3 of TS 23.304 [4]. The discovery between the ProSe UE-to-Network Relay and target UEs is triggered based on UE pre-configuration. Once discovered, the connections between the reference UE and the other UEs are established over PC5. It is not necessary to establish any UP PDU session for such special RSCs between the reference UE (or other UEs) and network as such RSCs may not be associated with any specific PDU session parameters.

9.	The ProSe application server (or PCF) responds or notifies the LMF when the relay configuration is complete.

10.	The LMF sends a ranging/sidelink positioning request to UE1. The request may contain multiple target UEs.

NOTE 1:	UE1 is assumed to be the located UE acting as reference UE, in coverage for this solution. UE1 is also acting as ProSe UE-to-Network Relay.

11	UE1 initiates side-link based positioning over PC5 for the multiple target UEs (as instructed by the LMF in step 11). This may include obtaining position estimates from a multiple UEs, which e.g. would carry out UE based positioning, obtain positioning assistance data from these UEs or execute a sidelink based positioning method (and gaining relative or absolute position information of the set of UEs). Whichever method is used, the reference UE will collect this data through the side-link in this step.

12.	UE1 sends the ranging/sidelink positioning measurements and/or results to the LMF. The location information from multiple UEs can be concatenated into a fewer positing protocol messages at the reference UE, in order to save signalling overhead. If requested by the server, the LMF may combine the location information of UE1 with the ranging/sidelink positioning information related to the target UEs to derive absolute location information for the target UEs.

13.	The LMF forwards the ranging/sidelink positioning and location results to the AMF.

14.	The AMF forwards the ranging/sidelink positioning and location results to the GMLC.

15.	The GMLC exposes the ranging/sidelink positioning and location results to the ranging application server via NEF with the response operation of the service employed in step 1.

### 6.12.4	Impacts on services, entities, and interfaces

GMLC

-	Support for ranging/SL positioning request for one or multiple UEs with corresponding events

LMF

-	Support for ranging/SL positioning request for one or multiple UEs with corresponding events

-	LMF selection considering support for ranging/SL positioning

-	LMF obtaining the UE's ProSe capability from AMF-	To support interactions with the ProSe Application Server and PCF.

AMF

-	To register the support for location reporting as part of ProSe capabilities from the UE.

UE

-	Support to send/receive ranging/SL positioning request to/from LMF

-	Support to request ranging/SL positioning service from 5GS

-	As a reference UE (similar to a ProSe UE-to-Network Relay) to be able to interpret RSCs for location reporting.

-	As a reference UE to be able to concatenate location information of other UEs as LPP messages towards 5GC.

-	As a target UE (similar to ProSe Remote UE) to be able to interpret RSCs for location reporting.

## 6.13	Solution #13: Ranging/SL positioning service initiated by application server or 5GC NF

### 6.13.1	General

This solution is to address KI#7.

A Ranging/SL positioning service request may be initiated by an application server for e.g. a commercial service or by a 5GC NF for e.g. network assisted sidelink positioning.

The service request is transmitted to one of the UEs, and the UE then initiates the Ranging/SL positioning procedure.

Once the result is calculated, the UE who receives the service request reports the result to the application server or 5GC NF who initiates the service request.

### 6.13.2	Functional descriptions

The service request and the result reporting can be transmitted over the control plane. The 5GC NF who initiates or transmits the service request needs to check with UDM to acquire the serving AMF of one of the UEs subject to Ranging/SL positioning. The service request is then sent to the serving AMF of one of the UEs, and the AMF forwards the service request to the UE over NAS message.

When UE receives the service request either from application server or from AMF, it initiates the Ranging/SL positioning procedure to the other UE for the measurement and calculation of the result.

NOTE:	Ranging/SL positioning procedure includes Ranging/Sidelink positioning discovery and service operations, which will be addressed in the solutions under KI#3 and KI#4.

The Ranging/SL positioning service request includes the following parameters:

-	Reference UE ID

-	Target UE ID

-	Result content: distance, angle or both

-	Required QoS

Editor's note:	Whether there are any other parameters in the Ranging/SL positioning service request is FFS.

### 6.13.3	Procedures

#### 6.13.3.1	Service initiated by application server

6.13.3.1-1: Service initiated by application server

1.	Service authorization policy/parameters are provisioned to UE1 and UE2.

2.	AF sends the Ranging/SL positioning service request to NEF, including UE1 ID, UE2 ID, Result content (distance, angle or both) and Required QoS, etc.

3.	NEF checks UDM to discover the serving AMF of UE1, and then forwards the request to the AMF.

NOTE 1:	NEF also checks the privacy on whether the AF is authorized to acquire Ranging/SL positioning information of UE1 and UE2. The solution will be developed by SA WG3.

NOTE 2:	UE1 can be either the target UE or the reference UE, which can be requested by the AF or can be decided during step 5.

Editor's note:	How the reference UE and target UE are decided is FFS.

4.	AMF forwards the service request to the UE1 over NAS.

5.	UE1 initiates Ranging/SL positioning procedure to UE2 to trigger the measurement and the calculation of the result. The Ranging/SL positioning procedure includes Ranging/Sidelink positioning discovery and service operations.

NOTE 3:	Solutions for Ranging/Sidelink positioning discovery and service operations will be developed under KI#3&4. Whether DDNMF will be involved depends on the solution.

6.	UE1 sends the Ranging/SL positioning result to the AMF.

7.	AMF forwards the Ranging/SL positioning result to the NEF.

8.	NEF forwards the Ranging/SL positioning result to the AF.

Editor's note:	It is FFS whether LMF or GMLC needs to be involved.

Editor's note:	It is FFS NEF/AMF performs any checking/authorization/processing.

#### 6.13.3.2	Service initiated by 5GC NF

6.13.3.2-1: Service initiated by 5GC NF

1.	Service authorization policy/parameters are provisioned to UE1 and UE2.

2.	5GC NF checks UDM to discover the serving AMF of UE1, and then sends the request to the AMF, including UE1 ID, UE2 ID, Result content (distance, angle or both) and Required QoS, etc.

NOTE 1:	5GC NF also checks the privacy on whether the AF is authorized to acquire Ranging/SL positioning information of UE1 and UE2. The solution will be developed by SA3.

NOTE 2:	UE1 can be either the target UE or the reference UE, which can be requested by the AF or can be decided during step 4.

Editor's note:	How the reference UE and target UE are decided is FFS.

3.	AMF forwards the service request to the UE1 over NAS.

4.	UE1 initiates Ranging/SL positioning procedure to UE2 to trigger the measurement and the calculation of the result. The Ranging/SL positioning procedure includes Ranging/Sidelink positioning discovery and service operations.

NOTE 3:	Solutions for Ranging/Sidelink positioning discovery and service operations will be developed under KI#3&4. Whether DDNMF will be involved depends on the solution.

5.	UE1 sends the Ranging/SL positioning result to the AMF.

6.	AMF forwards the Ranging/SL positioning result to the 5GC NF.

Editor's note:	It is FFS whether LMF or GMLC needs to be involved.

Editor's note:	It is FFS NEF/AMF performs any checking/authorization/processing.

### 6.13.4	Impacts on services, entities, and interfaces

The solution has impacts in the following entities and interfaces:

-	UE: supports initiating Ranging/SL Positioning procedure; receives Ranging/SL Positioning service request from AMF; sends Ranging/SL Positioning result to the AMF

-	AMF: receives and forwards the Ranging/SL Positioning service request to the UE1; receives and forwards the Ranging/SL Positioning result to the 5GC NF/NEF

-	NAS: supports transmission of Ranging/SL Positioning service request and Ranging/SL Positioning result

## 6.14	Solution #14: General reference architecture for Sidelink positioning and ranging-based services

### 6.14.1	General

Figure 6.14.1-1 shows a reference architecture for Sidelink Positioning and Ranging-based services for non-roaming operation. In this case, the UE A and UE B that are involved in Sidelink Positioning and Ranging-based services have subscription from the same PLMN. The reference architecture also supports the case that UE-A or UE-B or both are not registered to the network or not in coverage. UE C and UE D may be out of coverage, or with partial network coverage.

Editor's note:	Further impacts to the LCS NF are FFS.

Figure 6.14.1-1 Reference architecture for Sidelink Positioning and Ranging-based services for non-roaming and same PLMN operation

Figure 6.14.1-2 shows a reference architecture for Sidelink Positioning and Ranging-based services for Inter-PLMN non-roaming operation. In this case, the UE A and UE B have subscriptions from different PLMNs, i.e. PLMN-A and PLMN-B respectively. UE C and UE D may be out of coverage, or with partial network coverage.

Figure 6.14.1-2 Reference architecture for Sidelink Positioning and Ranging-based services for inter-PLMN operations

Figure 6.14.1-3 presented a reference architecture for Sidelink Positioning and Ranging-based services for Inter-PLMN non-roaming operation. In this case, the UE A and UE B have subscriptions from different PLMNs, i.e. PLMN-A and PLMN-B respectively. In addition, UE A is roaming and registered to PLMN-C. UE C and UE D may be out of coverage, or with partial network coverage.

Figure 6.14.1-3 Reference architecture for Sidelink Positioning and Ranging-based services for inter-PLMN operation with roaming

### 6.14.2	Functional descriptions

Sidelink Positioning and Ranging-based services are supported based on the architectures in clause 6.14.1, with the following reference points:

SR1:	The reference point between the UE Sidelink (SL) Positioning and Ranging application in the UE and the SL Positioning/Ranging Server. This reference point is out of scope of this specification. It may be used for the configuration and application layer signalling.

SR5:	The reference point between the Sidelink (SL) Positioning and Ranging application in UEs. It is carried over the PC5 reference point. The operation over this interface is used to control the Sidelink Positioning and Ranging operation over the PC5 reference point between the UEs.

Editor's note:	Whether SR5 also covers discovery operations for Sidelink Positioning and Ranging needs to be clarified.

PC5:	The reference point between the UEs, and it includes the LTE based PC5 and/or NR based PC5. It also supports the Sidelink Positioning and Ranging operation defined by RAN WGs.

Editor's note:	Reference to RAN WG specifications, e.g. TS 38.300 [8], to be added when it is available.

N1:	In addition to the relevant function defined in TS 23.501 [14], if the UE is in coverage, it may be also used to convey the SL Positioning and Ranging policy (SLPRP) (including service authorization) from AMF to the UE, and to convey the UE's capability from the UE to the AMF. if the LMF supports Sidelink Positioning and Ranging-base services, it may be also used to carry the signalling between UE and LMF, as defined in TS 23.273 [11].

N2:	In addition to the relevant functions defined in TS 23.501 [14] for N2, in the case of SL Positioning and Ranging-based service is supported by NG-RAN, it is also used to convey the SLPRP policy and parameters (including service authorization) from AMF to NG-RAN.

Uu:	The reference point between the UE and the NG-RAN.

Following service-based interfaces are used to support Sidelink Positioning and Ranging-based services:

Nlmf:	In addition to the relevant services defined in TS 23.273 [11], if the LMF supports SL Positioning and Ranging-based service, it may be used to provide service to other NFs related to it.

Nudm:	In addition to the relevant services defined in TS 23.501 [14] for Nudm, in the case of SL Positioning and Ranging-based service, services provided by UDM are used to get the related subscription information to AMF during Initial registration procedure or UE Configuration Update (UCU) procedure to inform AMF subscription information has changed.

Npcf:	In addition to the relevant services defined in TS 23.501 [14] for Npcf, in the case of SL Positioning and Ranging-based service, services provided by H-PCF are used to provide V2X Service related parameters to V-PCF for UE and NG-RAN in the roaming case.

Nudr:	In addition to the relevant services defined in TS 23.501 [14] for Nudr, in the case of SL Positioning and Ranging-based service, services provided by UDR are used to notify the PCF and the UDM of the update of the SL Positioning and Ranging-based service related information.

Namf:	In addition to the relevant services defined in TS 23.501 [14] for Namf, in the case of SL Positioning and Ranging-based service, services provided by AMF are consumed by PCF to provide the SL Positioning and Ranging-based service related parameters for the UE and the NG-RAN to AMF, and to enable the AMF create or update UE context related to SL Positioning and Ranging-based service.

NOTE:	N5g-ddnmf services may be used when UE supports 5G ProSe Restricted discovery.

Editor's note:	Other options for maintaining the mapping between UE SUPI and UE Application Layer User ID is FS.

### 6.14.3	Procedures

UE A, UE B, UE C, and UE D may be pre-configured (in UICC or ME) or provisioned by PCF with the SL Positioning and Ranging-based service Policies (SLPRP). The UEs may also be configured by SL Positioning Ranging Server regarding the corresponding operation parameters and configurations via SR1.

UE A, UE B, UE C, and UE D carry out the discovery and control signalling for Sidelink Positioning and Ranging-based services over reference point SR5, which is carried over PC5 reference point for direct communications between the UEs. There may be more than two UEs involved in the Ranging-based service and Sidelink Positioning session, depending on the signalling control carried out over SR5.

When UE A or UE B are served by NG-RAN for Ranging-based services and Sidelink Positioning, the UE A and UE B may communicate with LMF, re-using enhanced procedures defined TS 23.273 [11]. The UE A (or UE B) and the LMF use a Sidelink Positioning Protocol to perform the operation.

UE C and UE D can operate out of coverage, or with partial network coverage. Sidelink Positioning and Ranging services can be performed without the network involvement.

UE A, UE B, UE C, and UE D carry out the necessary measurement for the Sidelink Positioning and Ranging-based service over the PC5 reference points. Detail operation of this step is defined by RAN WGs.

Depends on the SL Positioning and Ranging-based service operation negotiated during the control signalling, UE A, UE B, UE C, and UE D may exchange the measurement results or Sidelink Positioning or Ranging results over the SR5 reference point. When LMF is used, the measurement results may be sent to the LMF.

### 6.14.4	Protocol stack

Figure 6.14.4-1 illustrates an example UE protocol stack and the potential interactions of the function elements and other layers of the UE.

Figure 6.14.4-1: UE functions and architecture for sidleink positioning and ranging service

The function elements in figure 6.14.4-1 are defined as following:

-	Device and Service Discovery Function (DSDF): Discover and identify the UE(s) in proximity that can participate in Sidelink Positioning and Ranging service sessions, and it also provides to the upper layer about the service supported by each of the identified UE.

-	Group Support Service Function (GSSF): Perform group management requested by upper layer using the PC5 communication service provided by ProSe Layer or V2X layer. The messages generated by GSSF are sent as data traffic over the PC5 reference point and does not require additional enhancements to PC5 reference point.

-	Sidelink Positioning and Ranging Function (SPRF): Perform control signalling between UEs or among a group of UEs to manage and coordinate the Sidelink Positioning and Ranging operations. The messages generated by SPRF are sent as data traffic over the PC5 reference point and does not require additional enhancements to PC5 reference point.

In order to be supported over PC5, the three services, i.e. DSDF, GSSF, and SPRF, should obtain a V2X Service Type (e.g. PSID, ITS-AID, or AID) or a ProSe Application ID. It is possible for the three services to share the same identifier or separate identifiers.

## 6.15	Solution #15: Service Authorization to NG-RAN for Ranging/Sidelink Positioning

### 6.15.1	General

This solution is to address KI#8.

Since Ranging/Sidelink positioning is applied for V2X, public safety and commercial use cases and all Ranging/SL positioning capable UEs are 5G ProSe or V2X capable, it is assumed that Ranging/Sidelink positioning is always used together with 5G ProSe or V2X or both service(s). It can be seen as the extended capability of 5G ProSe or V2X and will be authorized by 5GC for the usage in the PLMN.

Solutions on Service Authorization to NG-RAN for V2X communications over PC5 reference point and for ProSe based service over PC5 reference point have been respectively defined in TS 23.287 [3] and TS 23.304 [4], however, they didn't consider the support of Ranging/Sidelink positioning.

This solution provides enhancements to mechanism of Service Authorization to NG-RAN for V2X communications over PC5 reference point and for ProSe based service over PC5 reference point for the support of Ranging/Sidelink positioning.

### 6.15.2	Functional descriptions

During registration, the UE includes the 5G ProSe Capability/V2X capability as part of the "5GMM capability" in the Registration Request message. The AMF stores the 5G ProSe Capability/V2X capability in the UE context.

The extension to the 5G ProSe capability or V2X capability due to the support of Ranging/Sidelink positioning includes:

-	Ranging/Sidelink positioning reference UE.

-	Ranging/Sidelink positioning target UE.

-	Ranging/Sidelink positioning assistant UE.

-	Ranging/Sidelink positioning Located UE.

During registration, AMF determines whether the UE is authorised to use 5G ProSe/V2X services based on UE's 5G ProSe/V2X Capability and the ProSe/V2X Service Authorisation included in the subscription data received from UDM. The AMF stores the authorized 5G ProSe Capability/authorized V2X Capability. The extension to the authorized 5G ProSe capability / authorized V2X capability due to the support of Ranging/Sidelink positioning includes:

-	Ranging/Sidelink positioning reference UE.

-	Ranging/Sidelink positioning target UE.

-	Ranging/Sidelink positioning assistant UE.

-	Ranging/Sidelink positioning Located UE.

The ProSe/V2X Service Authorisation of the subscription data also needs to be extended, which can be achieved by extending ProSe/V2X subscription data to include Ranging/Sidelink positioning information, see table 6.15.2-1.

Table 6.15.2-1: Extending ProSe/V2X subscription data

| V2X Subscription data (see TS 23.287 [3]) | NR V2X Services Authorization | Indicates whether the UE is authorized to use the NR sidelink for V2X services as Vehicle UE, Pedestrian UE, or both, and whether the UE is authorized to use or serve as Ranging/Sidelink positioning reference UE, target UE, assistant UE, Located UE or any combination of these roles. |
| --- | --- | --- |
|  | LTE V2X Services Authorization | Indicates whether the UE is authorized to use the LTE sidelink for V2X services as Vehicle UE, Pedestrian UE, or both and whether the UE is authorized to use LTE sidelink for Ranging/Sidelink positioning service control signalling. |
|  | NR UE-PC5-AMBR | AMBR of UE's NR sidelink (i.e. PC5) communication for V2X services. |
|  | LTE UE-PC5-AMBR | AMBR of UE's LTE sidelink (i.e. PC5) communication for V2X services. |
| ProSe Subscription data (see TS 23.304 [4]) | ProSe Service Authorization | Indicates whether the UE is authorized to use ProSe Direct Discovery, ProSe Direct Communication, or both, whether the UE is authorized to use or serve as a ProSe UE-to-Network Relay and whether the UE is authorized to use or serve as Ranging/Sidelink positioning reference UE, target UE, assistant UE, Located UE, or any combination of these roles. |
|  | ProSe NR UE-PC5-AMBR | AMBR of UE's NR sidelink (i.e. PC5) communication for ProSe services. |

If the UE is authorised to use 5G ProSe/V2X services, then the AMF shall include in a NGAP message sent to NG-RAN the "5G ProSe authorised"/"V2X authorised" information. The "5G ProSe authorised" information and "V2X authorised" information can be extended to include the Ranging/Sidelink positioning authorization information:

-	whether the UE is authorized to act as a Ranging/Sidelink positioning reference UE;

-	whether the UE is authorized to act as a Ranging/Sidelink positioning target UE;

-	whether the UE is authorized to act as a Ranging/Sidelink positioning assistant UE;

-	whether the UE is authorized to act as a Ranging/Sidelink positioning Located UE.

The Ranging/Sidelink positioning authorization information is provided as part of "5G ProSe authorised"/"V2X authorised" information from AMF to the NG-RAN node over NGAP during Registration procedure, Service Request procedure and N2 Handover procedure, and from source NG-RAN node to target NG-RAN node over Xn during Xn Handover procedure as defined in clause 6.5 of TS 23.287 [3] and clause 6.6 of TS 23.304 [4].

The Ranging/Sidelink positioning authorization information is only provided when scheduled resource allocation mode is utilized.

### 6.15.3	Procedures

Procedures specified in clause 6.5 of TS 23.287 [3] and clause 6.6 of TS 23.304 [4] are reused with the extension described in clause 6.15.2.

### 6.15.4	Impacts on services, entities, and interfaces

The solution has impacts in the following entities and interfaces:

-	UE: extends 5G ProSe Capability/V2X capability with Ranging/Sidelink positioning related parameters.

-	UDM: extends 5G ProSe /V2X subscription data with Ranging/Sidelink positioning related parameters.

NG-RAN: resource allocation for Ranging/Sidelink positioning based on the authorization

## 6.16	Solution #16: Solution for ranging operation with assistant UE(s)

### 6.16.1	General

This solution resolves Key Issue #2 for Ranging service operation procedure with the assistance of other UEs (e.g. assistant UEs). In particular, this solution presents the procedure of the ranging operation with the assistant UE(s).

### 6.16.2	Functional descriptions

In general, this solution proposes the principles in the ranging operation with the assistant UE(s):

1.	The direct ranging procedure defined by the solutions for KI#4 can be directly used for Assistant UE(s) to get the relative position of the UE1 and UE2 relative to the Assistant UE(s) respectively.

2.	UE1 as the initiator UE performs the ranging result calculation based on the relative position of the UE1 and UE2 relative to the Assistant UE(s).

### 6.16.3	Procedures

Figure 6.16.3-1: High-level procedure for Ranging operation with assistant UE

1.	UE1 and UE2 get the ranging authorization policy from PCF during the registration procedure. The ranging authorization policy may include whether the UE is authorized to perform ranging operation with assistant UE. UE3 (i.e. Assistant UE) gets the ranging authorization policy from PCF during the registration procedure. The ranging authorization policy may include whether the UE is authorized to act as an assistant UE for ranging operation.

2.	When the UE1 gets the ranging request from the application layer, but the direct ranging operation between the UE1 and UE2 cannot be supported, so UE1 will perform assistant UE discovery procedure. Specifically, there are two options:

Option A (Assistant UE selection by UE2): the UE1 sends a discovery message including the ranging indication and UE2 User Info, and if the UE receives the message including the ranging indication and supports as Assistant UE based on the authorization policy in step 1 or its ranging capability, it will generate and send a discovery message including UE2 User Info. After UE2 performs the Assistant UE selection based on e.g. signal strength or whether LOS path exists, and responses to the selected Assistant UE(s). The selected Assistant UE(s) further responses to the UE1.

Option B (Assistant UE selection by UE1): the UE1 sends a discovery message including the ranging indication and UE2 User Info, and if the UE receives the message including the ranging indication and supports as Assistant UE based on the authorization policy in step 1 or its ranging capability, it will generate and send a discovery message including UE2 User Info. When UE2 receives the discovery message from Assistant UE(s), UE2 responses to these Assistant UE(s), and these Assistant UE(s) further responses to UE1. UE1 performs the Assistant UE selection based on e.g. signal strength or whether LOS path exists.

NOTE 1:	The outcome of KI#1 "Support of UE-to-UE Relay" of FS_5G_ProSe_Ph2 described in TR 23.700-33 [18] may be considered to down-select Option A or Option B for a consistent UE action.

3.	UE1 sends the ranging request to the UE2, and the ranging request includes Assistant UE ID(s). Ranging request is sent over PC5 direct connection or UE-to-UE Relay connection.

When the ranging request from the application layer includes Ranging accuracy, then UE1 can determinate the Ranging accuracy 1 for direct ranging between UE1 and Assistant UE and the Ranging accuracy 2 for direct ranging between UE2 and Assistant UE, by considering the Ranging accuracy. Then the ranging request includes Ranging accuracy 2.

NOTE 2:	Coordination with RAN WGs is needed for the determination of Ranging accuracy 1 and Ranging accuracy 2.

UE1 may get a scheduled ranging time from application layer or generate a scheduled ranging time locally. The ranging request includes the scheduled ranging time, and then step 4 and step 5 can be performed at the same scheduled ranging time.

4.	The UE1 obtains the relative position of the Assistant UE(s) (named as relative position 1), by triggering the direct ranging procedure defined by the solutions for KI#4 and using Ranging Method 1 based on Ranging accuracy 1. If multiple Assistant UEs are used, the direct ranging procedure can be performed for each Assistant UE.

5.	The UE2 obtains the relative position of the Assistant UE (named as relative position 2), by triggering the direct ranging procedure defined by the solutions for KI#4 and using Ranging Method 2 based on Ranging accuracy 2. If multiple Assistant UEs are used, the direct ranging procedure can be performed for each Assistant UE.

NOTE 3:	Coordination with RAN WGs is needed for the determination of Ranging Method 1/2 based on Ranging accuracy 1/2.

6.	UE2 sends the relative position 2 to the UE1.

NOTE 4:	UE-to-UE Relay connection used in step 3 and step 6 depends on the solutions for KI#1 "Support of UE-to-UE Relay" of FS_5G_ProSe_Ph2 described in TR 23.700-33 [18].

7.	UE1 performs the ranging result calculation according to the relative position 1 and the relative position 2.

### 6.16.4	Impacts on services, entities, and interfaces

UE:

-	The assistance UE(s) discovery.

-	Interaction for ranging request and ranging result.

## 6.17	Solution #17: Ranging/Sidelink positioning with the assistance of assistant UE

### 6.17.1	General

This solution is to address KI#2.

When UE1 receives a service request for Ranging/Sidelink positioning between UE1 and UE2, it triggers Ranging/Sidelink operation. At the time UE1 triggers Ranging/Sidelink operation, UE1 doesn't know if direct communication between UE1 and UE2 is possible, no matter whether UE2 has been successfully discovered, because UE2 may move to a place where direct communication is not possible.

When UE1 sends out the service request, it expects that this service request is received by UE2 directly. If it can't be achieved, it has to sort help from the assistant UE who can assist the Ranging/Sidelink positioning between UE1 and UE2, i.e. who can perform Ranging/Sidelink positioning respectively with UE1 and UE2, and can transmit the measurement data in between.

If multiple assistant UEs are available, UE1 needs to discover and select one or multiple assistant UE to participate Ranging/Sidelink positioning between UE1 and UE2.

Ranging/Sidelink positioning is performed between one or multiple assistant UE and UE1 and between one or multiple assistant UE and UE2.

The final result of Ranging/Sidelink positioning between UE1 and UE2 can be derived from the measurement data of Ranging/Sidelink positioning between assistant UE and UE1 and the measurement data of Ranging/Sidelink positioning between assistant UE and UE2.

### 6.17.2	Functional descriptions

An assistant UE has the following functionality:

-	Ranging/Sidelink positioning measurement.

-	Relay Ranging/Sidelink positioning measurement data and/or result between UE1 and UE2.

-	Ranging/Sidelink positioning result calculation (optional).

To discover and select an assistant UE, UE1 broadcast the Ranging/Sidelink positioning request. UE1 includes in the Ranging/Sidelink positioning request an indication of "assistant UE allowed" in addition to the other parameters (e.g. UE1 ID and UE2 ID, Ranging/Sidelink positioning content (i.e. distance, direction) QoS requirement). "Assistant UE allowed" indicates that Ranging/Sidelink positioning with the assistance of another UE is acceptable.

NOTE 1:	The UE1 and UE2 ID may be application layer ID.

When the broadcast Ranging/Sidelink positioning request is received over PC5, the potential assistant UE matches with the UE2 ID. If UE2 can receive the request directly, it responds to UE1. If the potential assistant UE is authorized as the Ranging/Sidelink positioning assistant UE at the current PLMN, time and location by 5GC and when it receives from the Ranging/Sidelink positioning request the "Assistant UE allowed" indication, it checks locally if UE2 is already discovered. The potential assistant UE may maintain a list of UEs that are discovered by itself. In that case, the potential assistant UE searches from the list for UE2. If UE2 is already discovered, the potential assistant UE responds to UE1; otherwise, the potential assistant UE triggers UE2 discovery procedure and responds to UE1 when UE2 is successfully discovered.

NOTE 2:	If UE2 is also authorized as the Ranging/Sidelink positioning assistant UE at the current PLMN, time and location by 5GC, it responds to UE1 as UE2.

UE1 may receive response from UE2 directly, and/or one or multiple responds from assistant UE(s):

-	if UE1 receives response from UE2 directly, it initiates Ranging/Sidelink positioning procedure to UE2

-	Otherwise, if UE1 receives one or multiple responds from assistant UE(s), UE1 selects one or multiple assistant UE who sends the response first.

Since UE1 prefers direct Ranging/Sidelink positioning with UE2 than using assistant UE, UE1 may wait for a while to receive response from UE2 after a respond is received from any assistant UE. A timer may be configured at UE1. When time is up, if no respond from UE2 is received, UE1 determines to use assistant UE. The timer may be started when the first response is received from any assistant UE or may be started when Ranging/Sidelink positioning request is sent.

When Ranging/Sidelink positioning uses assistant UE and the assistant UE is determined, UE1 initiates Ranging/Sidelink positioning procedure to the selected assistant UE. An assistant UE ID is included in the request in addition to other parameters (e.g. UE1 ID and UE2 ID, Ranging/Sidelink positioning content (i.e. distance, direction) QoS requirement) to indicate that Ranging/Sidelink positioning between UE1 and UE2 will be performed with the assistance of the assistant UE.

When the selected assistant UE receives Ranging/Sidelink positioning request which contains its own ID as the assistance UE ID, it performs Ranging/Sidelink positioning with UE1, initiates the Ranging/Sidelink positioning to UE2, including in the request UE1 ID and assistance UE ID, and performs Ranging/Sidelink positioning with UE2.

The measurement data/result will be shared among UE1, UE2 and assistant UE to derive the Ranging/Sidelink positioning result, and one of UE1, UE2 and assistant UE calculates the result based on the measurement data/result.

Service authorization and policy/parameter provisioning take place during registration or UE triggered Policy provisioning. A UE capable of being Ranging/Sidelink positioning assistant UE indicates the capability in the UE Policy Container to the PCF. The PCF authorizes the UE as Ranging/Sidelink positioning assistant UE based on the capability received in the UE Policy Container and other inputs.

NOTE 3:	Details on how a UE is authorized as an assistant UE is developed in solutions for KI#1.

### 6.17.3	Procedures

Figure 6.17.3-1 Procedure of Ranging/Sidelink positioning with the assistance of assistant UE

1.	UE1 and UE2 are authorized as Ranging/Sidelink positioning reference/target UE at the current PLMN, time and location. Assistant UE A, assistant UE B, and Assistant UE C are authorized as Ranging/Sidelink positioning assistant UE at the current PLMN, time and location.

2.	UE1 receives a Ranging/Sidelink positioning request from is application layer, 3rd party UE or 5GC NF for Ranging/Sidelink positioning between UE1 and UE2.

3.	UE1 broadcasts the Ranging/Sidelink positioning request, which includes an indication of "assistant UE allowed" in addition to the other parameters (e.g. UE1 ID and UE2 ID, Ranging/Sidelink positioning content (i.e. distance, direction) QoS requirement).

4.	When receiving Ranging/Sidelink positioning request, which includes an indication of "assistant UE allowed" and UE2 ID, assistant UE A, assistant UE B, and Assistant UE C check locally if UE2 is already discovered. If not, they start to discover UE2 respectively, the discovery procedure is performed, during which UE2 is discovered; otherwise this step is skipped.

NOTE 1:	Details of the discovery procedure will be developed in solutions for KI#3.

5.	Assistant UE A successfully discovers UE2, and sends the Ranging/Sidelink positioning response to UE1.

6.	When Ranging/Sidelink positioning response is received from Assistant UE A, UE1 may start a preconfigured timer. When time is up and no response is received from UE2, UE1 determines to use Assistant UE A for Ranging/Sidelink positioning.

7.	UE1 initiates Ranging/Sidelink positioning procedure to assistant UE A. An assistant UE ID is included in the request in addition to other parameters (e.g. UE1 ID and UE2 ID, Ranging/Sidelink positioning content (i.e. distance, direction) QoS requirement) to indicate that Ranging/Sidelink positioning between UE1 and UE2 will be performed with the assistance of the assistant UE. Ranging/Sidelink positioning is performed between UE1 and assistant UE A.

Editor's note:	The security issue, e.g. whether the selected assistant UE is allowed to participate the Ranging/Sidelink positioning between UE1 and UE2 is FFS, which will be evaluated by SA3.

8.	When assistant UE A receives Ranging/Sidelink positioning request which contains its own ID as the assistance UE ID, it initiates the Ranging/Sidelink positioning to UE2, and includes in the request UE1 ID and UE A ID as the assistance UE ID. Ranging/Sidelink positioning is performed between UE2 and assistant UE A.

9.	The measurement data/result are shared among UE1, UE2 and assistant UE A to derive the Ranging/Sidelink positioning result.

NOTE 2:	Whether the calculation is performed at UE1, UE2, assistant UE A or any combination of them should be coordinated among them.

Editor's note:	Whether there is any security concern of sharing the measurement data/result to the selected assistant UE is FFS, which will be evaluated by SA3.

10.	UE1 responds with the Ranging/Sidelink positioning result.

### 6.17.4	Impacts on services, entities, and interfaces

The solution has impacts in the following entities and interfaces:

-	UE:

-	receives Ranging/Sidelink positioning request from is application layer, 3rd party UE or 5GC NF for Ranging/Sidelink positioning.

-	sends Ranging/SL Positioning result to application layer, 3rd party UE or 5GC NF.

-	broadcasts the Ranging/Sidelink positioning request, which includes an indication of "assistant UE allowed" in addition to the other parameters.

-	handle Ranging/Sidelink positioning request, which includes an indication of "assistant UE allowed".

-	handle Ranging/Sidelink positioning request, which includes an assistant UE ID.

-	PC5: supports transmission of Ranging/SL Positioning service request and Ranging/SL Positioning result.

## 6.18	Solution #18: Network triggered ranging device discovery

### 6.18.1	Description

This solution is related to the KI#3: Ranging/Sidelink Positioning device discovery and mainly focuses on the following sub-KI:

-	How Ranging/Sidelink Positioning devices' discovery is triggered at UE based on network instruction for the cases of in coverage, partial coverage and out of coverage?

From the low power consumption consideration, the ranging devices constraint on the battery capability may not always perform the ranging device discovery and ranging measurement over the sidelink/PC5. No ranging service requirement for a long time may affect that the device turn off or de-activate the ranging device discovery and ranging measurement. If current the peer device (UE#2) has ranging requirement with UE#1, direct discovery over PC5 seems not work. In this regard, it is proposed to use the Network triggered ranging service discovery to activate the peer UE device discovery procedure.

In this solution, for the network triggered ranging device discovery, as described in figure 6.18.1-1, the UE#1 sends the ranging device discovery request to the network, then the network notifies that the UE#2 turns on or activate the ranging device discovery. After receiving the notification from the network, UE#2 initiates the ranging device discovery with UE#1.

This solution has 2 alternatives as following:

-	5GC based network triggered ranging device discovery.

-	Application layer-based Network triggered ranging device discovery.

For the 5GC-based Network triggered ranging device discovery, the above-mentioned procedure is performed over the Control plane as descried in clause 6.18.2 and this procedure is designed for UEs from same PLMN.

For the Application layer-based Network triggered ranging device discovery, the above-mentioned procedure is performed over the application layer as described in clause 6.18.3, which may be not in the SA2 scope for some interactions. This procedure can work for UEs from different PLMN.

Figure 6.18.1-1: High-level procedure for Network triggered ranging device discovery

### 6.18.2	Control plane-based Network triggered ranging device discovery

#### 6.18.2.1	General

For the Control plane-based Network triggered ranging device discovery, this solution provides to 2 procedures as following:

-	One-shot Control plane-based Network triggered ranging device discovery procedure.

-	Deferred Control plane-based Network triggered ranging device discovery procedure.

For the one-shot procedure, the network notifies that the UE#2 activates the ranging procedure after receiving request from the UE#1, the proximity is determined by UE#1.

For the Deferred procedure, the UE#1 may subscribe to the network to monitor the proximity between UE#1 and UE#2, once the proximity is determined, the network notifies that the UE#1 and UE#2 activate the ranging device discovery.

#### 6.18.2.2	One-shot Control plane-based Network triggered ranging device discovery

Figure 6.18.2.2-1: One-shot Control plane-based Network triggered ranging device discovery procedure

1.	UE#1 sends a Ranging device discovery request to the AMF to invoke the Network triggered ranging device discovery procedure for activating/turning on the ranging discovery of UE#2, the Ranging device discovery request is transferred via the UL NAS transport message including the UE#2 ID and indication of one-shot procedure or deferred procedure.

2.	Considering the privacy, the AMF may check subscriber data of the UE#1 or UE#2 whether the UE#1 is authorized to trigger the UE#2 to activate the ranging service.

3.	If authorized in step2, the AMF selects a LMF supporting Network assistant ranging/sidelink positioning procedure.

4.	AMF send the Ranging device discovery request to the selected LMF along with the UE#2 ID and indication of one-shot procedure or deferred procedure.

5.	After receiving above request from the AMF, LMF may determine the role(announcer/monitor) of UE#1 and UE#2 in ranging device discovery. And then the LMF sends a Ranging device activation request including the role of UE#2 and UE#1 ID. LMF determine one-shot procedure in the next steps or deferred procedure as in clause 6.18.2.3 based on the indication in above request.

6.	UE#2 determine to activate the ranging device discovery with UE#1 after receiving request from LMF. UE#2 performs the device discovery procedure (announcing or monitoring the discovery message) based on role provided by LMF.

7.	UE#2 responses to the LMF that it has activated its ranging device discovery.

8.	LMF responses to AMF with a ranging device discovery response message including the role of UE#1.

9.	AMF forwards the ranging device discovery response message from LMF to the UE#1 that UE#2 has activated the ranging device discovery.

10.	The UE#1 performs the Ranging device discovery (announcing or monitoring the discovery message) with UE#2 based on role provided by LMF.

Editor's note:	It is FFS how to handle the different AMF and LMF case.

#### 6.18.2.3	Deferred Control plane-based Network triggered ranging device discovery

Figure 6.18.2.3-1: Deferred Control plane-based Network triggered ranging device discovery procedure

1.	UE#1 sends a Ranging device discovery subscribe request to the AMF which requests the network to monitor the proximity between UE#1 and UE#2, once the proximity is determined, the network notifies that the UE#1 and UE#2 activate the ranging device discovery.

2.	Considering the privacy, the AMF may check subscriber data of the UE#1 or UE#2 whether the UE is authorized to trigger the UE#2 to activate the ranging service.

3.	If authorized in step2, the AMF selects a LMF supporting Network assistant ranging/sidelink positioning procedure.

4.	AMF send the Ranging device discovery subscribe request to the selected LMF along with the UE#1 ID and UE#2 ID.

5.	After receiving above request from the AMF, LMF may determine the role(announcer/monitor) of UE#1 and UE#2 in ranging device discovery and determine the period of performing the positioning procedure with UE#1 and UE#2. LMF responds to the AMF the Ranging device discovery subscribe response.

6.	AMF forwards the ranging device subscribe discovery response message from LMF to the UE#1.

7.	LMF performs the UE positioning as described in TS 23.273 [11] with UE#1 and UE#2 periodically.

8.	LMF determines whether the UE#1 and UE2 are proximate based on the positioning results of UE#1 and UE#2.

9.	If the proximity is detected, the LMF notifies that the UE#1 and UE#2 activate the Ranging device discovery.

10.	After receiving notification from LMF, the UE#1 and UE2 activate the Ranging device discovery to discover each other.

NOTE:	UE#1 and UE#2 performs the device discovery procedure (announcing or monitoring the discovery message) based on role provided by LMF.

Editor's note:	It is FFS how to handle the different AMF and LMF case.

### 6.18.3	Application layer-based Network triggered ranging device discovery

#### 6.18.3.1	General

For the Application layer-based Network triggered ranging device discovery, similarly, this solution provides to 2 procedures as following:

-	One shot Application layer-based Network triggered ranging device discovery procedure

-	Deferred Application layer-based Network triggered ranging device discovery procedure

For the one-shot procedure, the AF notifies that the UE#2 activates the ranging procedure after receiving request from the UE#1, the proximity is determined by UE#1.

For the Deferred procedure, the UE#1 may subscribe to the AF to monitor the proximity between UE#1 and UE#2, once the proximity is determined, the network notifies that the UE#1 and UE#2 activate the ranging device discovery.

#### 6.18.3.2	One shot Application layer-based Network triggered ranging device discovery

Figure 6.18.3.2-1: One shot Application layer-based Network triggered ranging device discovery

1.	UE#1 may send the application message of ranging device discovery including the UE#2 ID to request the AF to activate the ranging device discovery of UE#2.

2.	AF performs the privacy check whether the UE#1 is authorized to perform ranging service with UE#2. AF sends the ranging device discovery request including UE#1 ID and UE#2 ID to NEF.

Editor's note:	Whether to perform privacy check needs coordination with SA3.

3.	NEF check the UE#1 and UE#2 subscribe data

4.	Based on the subscribe data, the NEF determines the authorization result.

5.	NEF returns the authorization result to AF via the ranging device discovery response.

6.	If it is authorised, the AF sends the application message of ranging device discovery request to UE#2.

7.	After receiving the request from AF, the UE#2 activates the ranging device discovery with UE#1.

8.	UE#2 responds to AF.

9.	AF responds to UE#1.

10.	UE#1 initiates the ranging device discovery with UE#2.

#### 6.18.3.3	Deferred Application layer-based Network triggered ranging device discovery procedure

Figure 6.18.3.3-1: Deferred Application layer-based Network triggered ranging device discovery procedure

1.	UE#1 sends a Ranging device discovery subscribe request over application message to the AF which requests the AF to monitor the proximity between UE#1 and UE#2, once the proximity is determined, the AF notifies that the UE#1 and UE#2 activate the ranging device discovery.

2.	Considering the privacy, the AF may check subscriber data of the UE#1 or UE#2 whether the UE is authorized to trigger the UE#2 to activate the ranging service as described in steps 2-5 in figure 6.18.3.2-1.

3.	AF responds to UE#1 based on the privacy checking results in step 2.

4.	AF performs the Deferred 5GC-MT-LR Procedure as described in TS 23.273 [11] with UE#1 and UE#2.

5.	AF determines whether UE#1 and UE#2 are proximate.

6.	If proximate, the AF notifies the UE#1 and UE#2 of performing the ranging device discovery procedure.

7.	UE#1 and UE#2 perform the ranging device discovery to discover each other.

### 6.18.4	Impacts on Existing Nodes and Functionality

The solution has impacts on the following entities:

UE(s):

-	request the network to activate the ranging device discovery.

-	activate the ranging device discovery after receiving request from network.

AMF:

-	LMF selection.

-	privacy checking with UDM whether UE is able to trigger another UE to perform ranging service.

-	request LMF to notify ranging device discovery.

LMF:

-	supports to notify ranging device discovery via LPP message.

UDM:

-	has the subscription data whether UE is able to be triggered by another UE to perform the ranging service.

AF:

-	privacy checking with UDM whether UE is able to trigger another UE to perform ranging service.

-	activate the ranging device discovery.

## 6.19	Solution #19: Support of Sidelink Positioning and Ranging signalling over different PC5 RATs

### 6.19.1	General

This solution addresses the key issue#4 on " Control of Operations for Ranging/Sidelink positioning". Specifically, this solution focused on supporting the control signalling over different PC5 access technologies, with minimum AS layer impacts.

As described by solution#14 (clause 6.14), the system architecture supports interactions between the UEs (e.g. UE-A and UE C, UE A and UE B, or UE C and UE-D) and between UE and 5GC. For the interactions between the UEs, the operation over the SR5 reference point is carried over the PC5 reference point. Due to different deployment scenarios and operations, PC5 reference point may utilize different type of access technologies, including LTE PC5 and NR based PC5 for V2X use (as of TS 23.287 [3]), and 5G ProSe based PC5 (as of TS 23.304 [4]). For example:

-	Among V2X capable UEs, the control signaling defined for SR5 is carried out via LTE or NR PC5 (as defined in TS 23.287 [3]). The control signaling manages the actual AS layer Sidelink Positioning and Ranging signaling and measurements defined by RAN WGs.

-	Among 5G ProSe capable UEs, the control signaling for SR5 is carried out over 5G ProSe PC5 (as defined in TS 23.304 [4]). The control signaling manages the actual AS layer Sidelink Positioning and Ranging signaling and measurements defined by RAN WGs.

The function of the SR5 services, and the AS layer Sidelink Positioning and Ranging signaling and measurements defined by RAN WGs would not be affected by which PC5 RAT is used to carry the SR5 control signaling.

### 6.19.2	Functional descriptions

#### 6.19.2.1	Upper Protocol Layers

Figure 6.19.2.1-1 illustrates an example UE architecture and the potential interactions of the function elements and other layers of the UE for upper protocol layers. The architecture shown in Figure 6.19.2.1-1 is aligned with that for Solution #14 clause 6.14.4 but includes more details for each upper protocol layer.

Figure 6.19.2.1-1: UE functions and architecture for sidelink positioning and ranging service

The function elements identified in clause 6.19.1 are defined as follows:

-	Device and Service Discovery Function (DSDF):

-	DSDF provides the following services to the upper layer, e.g. Application Layer:

-	a unique identifier of the UE(s) in proximity that can participate in Sidelink Positioning and Ranging service;

-	services supported by each of the UE identified, e.g. signaling protocols, pairwise ranging, group ranging;

-	communication channel and sessions with the identified UEs.

-	DSDF may receive the following information from the upper layer:

-	trigger for the discovery of UEs to participate in Sidelink Positioning and Ranging services;

-	(optional) attributes of the UEs to be discovered;

-	indication of discovery by other UEs and corresponding attributes of the other UEs;

-	Request/permission for the corresponding Sidelink Positioning and Ranging services.

-	Operation performed by DSDF:

-	Performing device and service discovery using PC5 communication service provided by ProSe or V2X layer. For example, it may perform application layer discovery using broadcast service over PC5 reference point.

-	Group Support Service Function (GSSF):

-	GSSF may be part of the Application Layer (e.g. for V2X) or may be a layer below the Application layer (e.g. for MC services);

Editor's note:	it is FFS whether GSSF is in scope for 3GPP for V2X or whether GSSF for V2X remains out of scope and part of the Application Layer.

-	GSSF (when below the Application Layer) provides the following services to the upper layer, e.g. Application Layer:

-	creating a Sidelink Positioning and Ranging service group, when the upper layer has specified the group. GSSF needs to provide group ID and group local member IDs to the upper layer. GSSF needs to also manage the group members, e.g. addition or removal of group members, splitting or merge of groups, etc. as decided by the upper layer;

-	with upper layer application layer managed group function, e.g. V2X platooning application, the GMSF provides the needed support for the group member status monitoring, i.e. indication of group members of presence and capability/status of the group members. GSSF may make use of the service of DSDF for the needed operation.

-	GSSF may receive the following information from the upper layer:

-	request for the creation or deletion of a group containing identified UEs for Sidelink Positioning and Ranging service;

-	request for the operation of a specific group member, e.g. add or remove a group member;

-	request for the management of a group, e.g. merge 2 groups or split a group into 2 separate groups.

-	Operation performed by GSSF:

-	Performing group management requested by upper layer using the PC5 communication service provided by ProSe Layer or V2X layer. The messages generated by GSSF are sent as data traffic over the PC5 reference point and does not require additional enhancements to PC5 reference point.

NOTE 1:	The group support is only for the control signalling, e.g. to determine which UE should be part of the Sidelink Positioning and Ranging session. It does not affect how AS layer positioning/ranging signal operates.

-	Sidelink Positioning and Ranging Function (SPRF):

-	SPRF provides the following services to the upper layer, e.g. Application Layer:

-	on-demand Sidelink Positioning/Ranging, direction, or relative location of another UE or a specified group of UEs;

-	periodic Sidelink Positioning/Ranging, direction, or relative location of another UE or a specified group of UEs;

-	triggered Sidelink Positioning/Ranging, direction, or relative location of another UE or a specified group of UEs.

-	SPRF may receive the following information from the upper layer:

-	request for current range/direction or relative location of another UE or a specified group of UEs;

-	request for periodic range/direction or relative location of another UE or a specified group of UEs;

-	request for triggered range/direction or relative location of another UE or a specified group of UEs.

NOTE 2:	The request from upper layers may originate on the UE itself (e.g. due to a user request or in support of an autonomous UE function like driving assistance for V2X) or may be received by the UE from the network (e.g. an LMF) or from an external client.

-	Operation performed by SPRF:

-	Performing control signaling between 2 UEs or among a group of 2 of more UEs to manage and coordinate the Sidelink Positioning and Ranging operations. The messages generated by SPRF are sent as data traffic over the PC5 reference point and does not require additional enhancements to PC5 reference point.

-	If the network supports Sidelink Positioning and Ranging, SPRF exchange may happen between UE and the LMF (e.g. using NAS signaling), similar to that defined in TS 23.273 [11].

-	Service Exposure Function:

-	This function allows the UE to interact with the application layer, or the AF and external client.

-	If the network supports the Sidelink Positioning and Ranging, the interactions may happen between the UE and the LMF as defined in TS 23.273 [11].

-	The exact operation of service exposure to an external client or AF is out of scope of SA WG2.

In order to support the operation over different PC5 RATs, each of the SR5 service elements is assigned a V2X Service Type (e.g. PSID, ITS-AID, or AID as of TS 23.287 [3]) and a ProSe Identifier (e.g. Application ID as of TS 23.304 [4]). For example:

-	Device and Service Discovery Function (DSDF) is assigned PSID_ DSDF, and ProSe_AppID_DSDF;

-	Group Support Service Function (GSSF) is assigned PSID_GSSF, and ProSe_AppID_GSSF;

-	Sidelink Positioning and Ranging Function (SPRF) is assigned PSID_SPRF, and ProSe_AppID_SPRF.

NOTE 3:	It is also possible that all the three services are sharing the same V2X Service Type and ProSe Identifier.

#### 6.19.2.2	Support of V2X UE operations

For a V2X capable UE (as defined in TS 23.287 [3]), it would be configured with the corresponding information in the V2XP for the service types. The following element would be included in the Policy/Parameters for V2X communications over PC5 reference point, as in clause 5.1.2.1 of TS 23.287 [3]:

3)	Policy/parameters for PC5 RAT selection and for PC5 Tx Profile selection:

-	the mapping of PSID_DSDF/PSID_GSSF/PSID_SPRF to PC5 RAT(s) (e.g. LTE PC5, NR PC5 or both), and:

-	for LTE PC5, to the corresponding Tx Profiles (see TS 36.300 [19] for further information);

-	for NR PC5, to the corresponding NR Tx Profiles for broadcast and groupcast (see TS 38.300 [8] and TS 38.331 [20] for further information).

The other configuration parameters in clause 5.1.2.1 of TS 23.287 [3] may also apply accordingly. For example, when NR PC5 RAT is selected, the configuration may further determine the support of cast types, e.g.:

6)	Policy/parameters when NR PC5 is selected:

-	The mapping of PSID_DSDF/PSID_GSSF/PSID_SPRF to the default mode of communication (i.e. broadcast mode, groupcast mode or unicast mode).

In order to carry the SR5 signaling, the non-IP PDCP SDU type is utilized AS layers over the user plane PC5 reference point (as shown in clause 6.1.1 of TS 23.287 [3]).

Legend:

-	PC5-U: The SDAP/PDCP/RLC/MAC/PHY functionality is specified in TS 38.300 [8].

-	For PDCP SDU type "Non-IP", a "Non-IP Type" header included in the SDU by upper layer to indicate the type of non-IP messages carried will be specified in stage 3 specification.

Figure 6.19.2.2-1: User Plane Lower Protocol Layers for NR PC5 reference point

In this approach, the SR5 signaling is handled as V2X Application (with PSID_DSDF/PSID_GSSF/PSID_SPRF). The non-IP PDCP SDU type is supported by both LTE PC5 and NR PC5. Therefore, regardless of the RAT selected based on policy 3) above, the transport can be supported.

The Non-IP PDCP SDU contains a Non-IP Type header, which indicates the V2X message family used by the application layer. The non-IP Type header and allowed value is defined TS 24.587 [17]. To support the signalling for SR5, a new non-IP Type header value can be added, e.g. "SR5".

NOTE:	Alternatively, each of the three SR5 services, e.g. DSDF, GSSF, SPRF, can have its own non-IP Type header value.

With the use of non-IP PDCP SDU type, there is no need to change the AS layer for carrying the SR5 signaling.

When NR PC5 is selected, broadcast, groupcast, or unicast mode can be also supported without change of AS layer.

#### 6.19.2.3	Support of 5G ProSe UE operations

For a 5G ProSe capable UE, configuration regarding the SR5 signaling is configured using ProSeP, as defined in clause 5.1.3.1 of TS 23.304 [4]. For example, the policy could include the mapping of the corresponding ProSe Identifier for the SR5 services to the cast mode and the Layer-2 ID to use:

5)	Policy/parameters when NR PC5 is selected:

-	The mapping of ProSe services (e.g. ProSe_AppID_DSDF/ ProSe_AppID_GSSF/ ProSe_AppID_SPRF) to radio frequencies with Geographical Area(s).

-	The mapping of ProSe services (e.g. ProSe_AppID_DSDF/ ProSe_AppID_GSSF/ ProSe_AppID_SPRF) to Destination Layer-2 ID(s) for broadcast.

-	The mapping of ProSe services (e.g. ProSe_AppID_DSDF/ ProSe_AppID_GSSF/ ProSe_AppID_SPRF) to Destination Layer-2 ID(s) for groupcast.

-	The mapping of ProSe services (e.g. ProSe_AppID_DSDF/ ProSe_AppID_GSSF/ ProSe_AppID_SPRF) to default Destination Layer-2 ID(s) for initial signalling to establish unicast connection.

To carry the SR5 signaling messages over the PC5-U reference point as defined in clause 6.1.2 of TS 23.304 [4], there are two options:

-	Use the same non-IP PDCP SDU type as in the case of V2X UEs. Although 5G ProSe does not use the non-IP PDCP SDU type, the AS layer of 5G ProSe is shared with NR based V2X defined in TS 38.300 [8]. Therefore, the non-IP PDCP SDU type is supported without any AS layer impact.

-	Use the "unstructured" PDCP SDU type. This is supported by 5G ProSe UEs, and the separation of the DSDF, GSSF, SPRF signaling can be achieved with Destination L2 IDs as configured in ProSeP item 5) above.

### 6.19.3	Procedures

For a UE interested in Sidelink Positioning and Ranging service, it may invoke the DSDF service for discovery and to identify other UEs in proximity that support the operations. This may be based on the request from the upper layer.

Based on the pre-configuration, or provisioned policies and parameters as described in clause 6.19.2, the UE can determine the RAT (e.g. LTE PC5 or NR PC5) and corresponding Tx Profiles to be used for the DSDF, GSSF, and SPRF, based on V2X Service Type/ProSe Identifier to RAT mapping. Such configuration will also determine the communication modes to use, e.g. broadcast, groupcast, or unicast.

If it is determined that more than two UEs need to participate in the operation, the GMSF service is invoked to form the group or determine the group members' presence and status for the operation.

SPRF signaling is used for the control signaling between the UEs to determine the corresponding Sidelink Positioning and Ranging operation, e.g. the channel to use for the Sidelink Positioning or Ranging reference signal, the sequence and time slot for each of the UE to perform the signal transmission and measurements.

The UEs, e.g. UE A and UE B, or in other cases a group of UEs, carry out the Sidelink Positioning or Ranging reference signal transmission and measurements.

Based on the Sidelink Positioning or Ranging services negotiated by the SPRF signaling in the earlier phase, the UEs uses the SPRF signaling to exchange the measurement results. This enables the UEs to perform the calculation of the range/direction, or relative locations.

Depending on the SPRF signaling support, some UEs may also provide a location computation service to other UEs. When the network supports the Sidelink Positioning and Ranging service, a UE may send measurement results to an LMF, which would perform location computation on behalf of the UE.

### 6.19.4	Impacts on services, entities, and interfaces

UE supporting Sidelink Positioning and Ranging services need to support the following new service functions:

-	DSDF;

-	GSSF;

-	SPRF.

No change to the existing AS layer PC5 communication service.

NOTE 1:	Security protection of signaling for DSDF, GSSF, and SPRF needs to be reviewed by SA WG3.

V2X UE supports new non-IP header type value for SR5 signalling.

NOTE 2:	RAN WGs will determine the necessary AS layer enhancement for Sidelink Positioning or Ranging reference signal transmission and measurements.

NOTE 3:	New QoS parameters, e.g. default PPPP level or standardized PQI, may need to be specified for the DSDF, GSSF, SPRF signaling.

## 6.20	Solution #20: Network assisted Sidelink positioning with scheduled location time

### 6.20.1	General

This solution relates to the KI#5. The scheduled location time defined in TS 23.273 [11] is used to get the Ranging/Sidelink positioning measurement data and the location of the Located UE at a same time, to achieve more accurate positioning of the target UE.

### 6.20.2	Functional descriptions

This solution addresses KI#5 and the following principles are used:

-	The positioning target UE (UE1) is within the network coverage.

-	Both MT-LR procedure and MO-LR procedure as specified in clause 6.1 of TS 23.273 [11] are supported.

-	When the LMF received the Located UE ID from the UE1, the LMF triggers a new positioning procedure for the Located UE with a scheduled location time, and triggers the UE1 to perform Ranging/Sidelink positioning with the Located UE at the same scheduled location time.

-	The UE1 performs Ranging/Sidelink positioning with the Located UE based on the scheduled location time and provides the Ranging/Sidelink positioning measurements data to the LMF.

-	Based on the position of the Located UE and the Ranging/Sidelink positioning measurement data between UE1 and Located UE, the LMF can calculate the UE1 position in a more precise way.

### 6.20.3	Procedures

#### 6.20.3.1	MT-LR procedure for Network assisted Sidelink positioning

Figure 6.20.3.1-1: MT-LR procedure for Network assisted Sidelink positioning

1.	MT-LR procedure is triggered and UE1 LMF receives a location request for UE1 as specified in clause 6.1 of TS 23.273 [11]. The location request may include a scheduled location time (Time T).

2.	UE1 LMF decides to use the network assisted Sidelink positioning for UE1, for example, UE1 has the sidelink capability or UE1 is authorized to use sidelink positioning, or the requested LCS QoS is not satisfied by the previous LCS positioning procedure. UE1 LMF triggers the discovery of a Located UE for positioning assistance.

NOTE 1:	LMF can decide when to send the request of step 2 based on the Time T, so that the discovery time in step 3 will not be a long time compared to Time T.

3.	UE1 performs the Located UE discovery, using the procedure defined by the solutions for KI#3. UE2 is discovered and selected as the Located UE.

4.	The UE1 sends UE2 ID to UE1 LMF.

NOTE 2:	What the UE2 ID is and its security aspect need the coordination with SA WG3.

5.	UE1 LMF triggers the location procedure for UE2 and sends a Provide Location Request to GMLC. The request message includes the Time T.

6.	GMLC triggers the procedure for UE2 positioning, as specified in clause 6.1 of TS 23.273 [11], and gets the UE2 location at Time T.

7.	GMLC responses UE2 location to UE1 LMF.

8.	UE1 LMF sends UL Positioning message to the UE1 with the Time T.

NOTE 3:	Step 8 can be triggered at the same time of step 5.

9.	UE1 performs Ranging/Sidelink positioning procedure with UE2 defined by the solutions for KI#3, and obtains the Ranging/Sidelink positioning measurement data at the Time T.

10.	UE1 provides the Ranging/Sidelink positioning measurement data to the UE1 LMF.

11.	UE1 LMF determines the UE1 location based on the Ranging/Sidelink positioning measurement data received in step 10 and UE2 location received in step 7.

#### 6.20.3.2	MO-LR procedure for Network assisted Sidelink positioning

Figure 6.20.3.2-1: MO-LR procedure for Network assisted Sidelink positioning

1.	UE1 performs the Located UE discovery for positioning assistance, using the procedure defined by the solutions for KI#3. UE2 is discovered and selected as Located UE.

2.	MO-LR procedure is triggered, and UE1 sends MO-LR Request to UE1 LMF, including UE2 ID and a scheduled location time (Time T).

NOTE 1:	What the UE2 ID is and its security aspect need the coordination with SA WG3.

3.	UE1 LMF triggers the location procedure for UE2 and sends a Provide Location Request to GMLC. The request message includes the Time T.

4.	GMLC triggers the procedure for UE2 positioning, as specified in clause 6.1 of TS 23.273 [11], and gets the UE2 location at Time T.

5.	GMLC responses UE2 location to UE1 LMF.

6.	UE1 LMF sends UL Positioning message to the UE1.

NOTE 2:	Step 6 can be triggered at the same time of step 3.

7.	UE1 performs Ranging/Sidelink positioning procedure with UE2 defined by the solutions for KI#3, and obtains the Ranging/Sidelink positioning measurement data at the Time T.

NOTE 3:	Step 7 can be triggered by UE1 itself based on the Time T.

8.	UE1 provides the Ranging/Sidelink positioning measurement data to the UE1 LMF.

9.	UE1 LMF determines the UE1 location based on the Ranging/Sidelink positioning measurement data received in step 8 and UE2 location received in step 5.

10.	UE1 LMF sends MO-LR Response to UE1 including the UE1 location.

### 6.20.4	Impacts on services, entities, and interfaces

UE:

-	Support Ranging and Sidelink positioning.

LMF:

-	Support the calculation of the UE location based on Ranging/Sidelink positioning measurement data and the location of the Located UE.

-	Trigger the Located UE positioning and Ranging/Sidelink positioning at same time.

## 6.21	Solution #21: Network assisted sidelink positioning for partial coverage

### 6.21.1	General

This solution is to address KI#5.

A UE out of coverage may be a UE not registered to the network or a UE temporarily unreachable. It may be in a known area, e.g. a factory or a campus, or not be in a known area, an application server may want to know its accurate location.

When a UE is out of coverage, positioning directly over Uu is not possible, communication between the UE and 5GC is not possible, either.

When 5GC determines that a UE is out of network coverage, it can consider to use Network assisted Sidelink Positioning to estimate UE location.

Since Network assisted Sidelink Positioning requires Ranging/Sidelink positioning between the UE and a Located UE, a Located UE needs to be determined. Located UE has to be in proximity of the UE out of coverage; however, UE should not be assumed to proactively discover a Located UE, because it doesn't know when a location service request comes.

### 6.21.2	Functional descriptions

A location service request for a UE may be sent to GMLC by an application server (via NEF) or a LCS client, and GMLC forwards the service request to an AMF. The serving AMF needs to be determined by the GMLC when UE is out of coverage.

UE out of coverage can be determined by GMLC or AMF, who will then triggers Network assisted Sidelink Positioning to locate a UE based on serving AMF address retrieval result or paging result:

Case 1:	When GMLC retrieves AMF address from the UDM with SUPI of this UE and no network address of the current serving AMF is returned

Case 2:	When UE's serving AMF identifies that UE is in CM-IDLE state and paging couldn't be successful

For Case 1, if Application server knows about the target area the UE is present, it provides in the LCS service request the target area to the GMLC, and the GMLC maps the target area into Cell ID/gNB ID/TAI and derives the serving AMF which can serve that area.

For Case 2, serving AMF is determined based on the successful retrieval of AMF address from the UDM.

The serving AMF of the UE/target area is responsible for discovery/selection the Located UE, which can be achieved in 2 steps:

1.	Provision of Located UE candidates: AMF may be preconfigured with list of Located UE, assuming those Located UE are nomadic, or if UE provides its capability of being Located UE during registration, AMF stores it into UE contexts. AMF selects Located UE candidates from the preconfigured list or based on UE capability.

2.	Selecting serving Located UE(s) from the list of Located UE candidates: If target area is provided, AMF selects one or multiple Located UE(s) based on the target area identified by Cell ID/gNB ID/TAI. If last known cell ID of the target UE is provided, AMF selects one or multiple Located UE(s) based on the cell ID.

The AMF then selects an LMF supporting Network assisted Sidelink Positioning as the serving LMF of the selected Located UE and sends the LCS service request to the LMF with providing UE ID and one or multiple Located UE ID.

When both UE ID and one or multiple Located UE ID are received, the LMF performs Network assisted Sidelink Positioning, including:

-	Discovery of the UE by the one or multiple Located UE over PC5. LMF can decide to stop discovery when one or multiple Located UE has reported successfully discovery of the UE.

-	Measurement and deriving UE location from:

-	Ranging/Sidelink positioning result/measurement data between UE and one or multiple Located UE.

-	Uu positioning result/measurement data of one or multiple Located UE.

### 6.21.3	Procedures

6.21.3.2-1: Procedure of network assisted sidelink positioning for partial coverage

0.	Located UE provides its Network assisted Sidelink positioning capability in the registration request to the AMF, and AMF stores it in the UE context.

1.	GMLC receives a LCS service request from the AF/LCS client, the request may include a target area the UE presents. GMLC checks service authorization and privacy with UDM.

2.	GMLC retrieves serving AMF of target UE from UDM. If no AMF address is returned and a target area is provided in the LCS service request, GMLC determines to use Network assisted sidelink positioning to locate the target UE. GMLC maps the target area into Cell ID/gNB ID/TAI, based on which derives the serving AMF which can serve that area as the serving AMF of the Located UE. If an AMF address is returned, GMLC selects the AMF as the serving AMF of the target UE.

3.	GMLC sends the LCS service request to the selected AMF, which includes the Cell ID/gNB ID/TAI of the target area if it is available.

NOTE:	When the AMF is derived based on Cell ID/gNB ID/TAI identifying the target area and multiple AMFs are derived, the LCS service request may be sent to some or all AMFs.

4.	If the UE is in CM IDLE state, the AMF initiates a network triggered Service Request procedure as defined in clause 4.2.3.3 of TS 23.502 [9] to establish a signalling connection with the UE. When the AMF is derived based on Cell ID/gNB ID/TAI identifying the target area, no UE state is available, this step is skipped.

5.	If the UE couldn't be successfully paged at step 4, AMF determines to invoke network assisted sidelink positioning to locate the UE. AMF selects one or multiple Located UE based on UE capability, cell ID/gNB ID/TAI of the target area, the last known cell ID of the target UE and the pre-configuration information.

6.	AMF selects a LMF capable of network assisted sidelink positioning.

7.	AMF sends the LCS service request to the LMF which includes target UE ID and one or multiple Located UE ID(s)

Editor's note:	The security issue, e.g. whether the selected list of Located UE is allowed to have the Ranging/SL positioning information of the target UE, is FFS, which will be evaluated in SA WG3.

8.	When both UE ID and one or multiple Located UE ID(s) are received in the LCS service request, LMF sends the Ranging/Sidelink positioning request to one or multiple Located UE(s) to trigger the Ranging/Sidelink positioning procedure.

9.	Located UE triggers the Ranging/Sidelink positioning procedure, which includes the discovery and service operation procedure over PC5.

10.	Located UE reports the Ranging/Sidelink positioning measurement or result to the LMF. LMF may receive measurement or result reporting from multiple Located UEs, LMF can determine whether to stop the further reporting from other Located UEs based on QoS requirement or pre-configuration.

11.	LMF triggers one or multiple procedures for the positioning of one or multiple Located UE. The LCS location estimation for the UE is generated based on the Ranging/Sidelink positioning result/measurement data between UE and one or multiple Located UE and Uu positioning result/measurement data of one or multiple Located UE.

12.	LMF provides the LCS location estimation of the UE to the GMLC.

13.	GMLC provides the LCS location estimation of the UE to the AF or LCS client.

### 6.21.4	Impacts on services, entities, and interfaces

The solution has impacts in the following entities and interfaces:

-	UE: supports Ranging/SL Positioning; provides capability of being Located UE to the AMF.

-	AMF: receives UE capability of Located UE and stores them; discovers and selects Located UE; determines to trigger network assisted sidelink positioning.

-	GMLC: determines serving AMF based on target area; determines to trigger network assisted sidelink positioning.

-	LMF: triggers the Ranging/Sidelink positioning procedure; derives UE location estimation based on Ranging/Sidelink positioning and Uu location measurements.

-	AF: provides target area in a LCS request.

## 6.22	Solution #22: Combine Uu positioning capability and/or Ranging/SL positioning capability for the Ranging/SL positioning service

### 6.22.1	General

This solution is related to the Key Issue #6 and Key Issue #7.

### 6.22.2	Functional descriptions

The LMF combines the Uu positioning capability and Ranging/Sidelink Positioning capability together to satisfy the Ranging/SL positioning service, which may improve the accuracy, enable Ranging/SL positioning service even when one UE lacks Uu/PC5 capabilities, enable Ranging/SL positioning service even when one UE is out-of-coverage, etc.

### 6.22.3	Procedures

Figure 6.22.3-1: Procedure for the Ranging/SL positioning service

1.	The AF (Step 1a) or 3rd party UE (Step 1b) requests the network for the Ranging/SL positioning service between the UE1 and UE2. The Ranging/SL positioning requirement may include the service requirement regarding Ranging/SL positioning demand (i.e. the distance and/or the direction between two UEs or more UEs), delay, accuracy, result feedback mode (e.g. immediately, periodically or at a later time). The UEs may be identified by GPSIs or SUPIs. If the GPSI is used, the AMF will request UDM to provide the associating SUPI.

2.	When the AMF receives the Ranging/SL positioning service request from UE or AF via GMLC, it sends Ranging/SL positioning Request (UE1 ID, UE2 ID, service requirement) to the LMF. The AMF is the serving AMF of 3rd party UE if the request is sent from the 3rd party UE, or the AMF is the serving AMF of the initiator UE (i.e. UE1/UE2, which is used to perform signalling exchange with the LMF, and the initiator UE may be selected by GMLC or indicated by AF) if the request is sent from AF.

3.	The LMF determines the appropriate positioning methods based on the service requirements, UE capability (e.g. whether UE has the Uu capability/PC5 capability), UE current status (e.g. reachable or unreachable, CM-CONNECTED or CM-IDLE) and operator policy. The positioning methods can be Uu positioning, Ranging/SL positioning, or both.

4.	The LMF initiates the Ranging/SL positioning between UE1 and UE2 if selected. The initiator UE reports the Ranging/SL positioning measurements to the LMF for calculation. When the Ranging/SL positioning calculation is performed by UE, the Ranging/SL positioning result is returned.

5a.	The LMF initiates Uu positioning for UE2 if selected.

5b.	The LMF initiates Uu positioning for UE1 if selected.

NOTE:	LMF also needs to query UDM to locate the AMFs of the UEs (UE-1 and UE2) in order to perform the positioning for both.

6.	The LMF performs the final calculation based on the Ranging/SL positioning result and/or Uu positioning result, i.e. calculates from UE1 Uu positioning result and UE2 Uu positioning result, when step 5a and step 5b are initiated; or calculates from UE1 Uu positioning result, UE2 Uu positioning result, Ranging/SL positioning result between UE1 and UE2, when step 5a, step 5b, and step 6 are initiated.

7.	The LMF sends Ranging/SL positioning Response (Ranging/SL positioning result) to the AMF.

8.	The AMF returns Ranging/SL positioning Service Response (Ranging/SL positioning result) to the AF (Step 8a) or 3rd party UE (Step 8b).

### 6.22.4	Impacts on services, entities, and interfaces

UE:

-	impacts in Sol#9 and Sol#12.

NG-RAN:

-	impacts in Sol#9 and Sol#12.

AMF:

-	impacts in Sol#9 and Sol#12.

LMF:

-	impacts in Sol#9 and Sol#12.

-	The LMF determines the appropriate positioning methods based on the service requirements, UE capability (e.g. whether UE has the Uu capability/PC5 capability), UE current status (e.g. reachable or unreachable, CM-CONNECTED or CM-IDLE) and operator policy.

## 6.23	Solution #23: Ranging service exposure to server

### 6.23.1	Description

This solution is related to the Key Issue #7 on Ranging/Sidelink Positioning service exposure to Application server and for network assisted sidelink positioning.

For the Ranging/Sidelink Positioning service exposure to Application server, it requires that the 5GS is able to provide the Ranging/Sidelink Positioning service to the Application server for obtaining the Ranging/Sidelink Positioning information (e.g. relative distance) between target UEs. To obtain this requirement, it needs the network assistance. In this solution, the network assistance can be reflected in the following aspects:

-	Determine the role, i.e. who is Reference UE or Target UE;

-	Provide the Ranging/Sidelink Positioning assistant data;

-	Obtain or calculate Ranging/Sidelink Positioning results.

This solution is more suitable to the case where:

-	the 2 UEs are both under network coverage,

-	the 2 UEs are unable to perform PC5 capability negotiation and Ranging/SL positioning calculation,

-	the 2 UEs supports Ranging/SL positioning capability negotiation with LMF;

### 6.23.2	Procedures of NW assistant Ranging service exposure to server

Figure 6.23.2-1: High-level procedure of Ranging/Sidelink Positioning with Model A discovery

This procedure basically takes the 5GC-MT-LR Procedure for the commercial location service as specified in clause 6.1.2 of TS 23.273 [11] as basis.

1.	AF (via NEF) sends a request to the GMLC for obtaining the Ranging/Sidelink Positioning information (e.g. relative distance) between 2 UEs with target UE's GPSI.

2.	The GMLC invokes the Nudm_SDM_Get service operation towards the UDM of the 2 UEs to get the privacy settings of the UE identified by its GPSI or SUPI. The UDM returns the target UEs Privacy setting of the UE. The (H)GMLC checks the UE Ranging/Sidelink Positioning privacy profile. If the 2 UEs are not allowed to be Ranged/Sidelink Positioned, the following steps 3-11 are skipped.

3.	The GMLC invokes a Nudm_UECM_Get service operation towards the UDM of the 2 UEs with GPSI or SUPI of the 2 UEs. The UDM returns the network addresses of the current serving AMF of the 2 UEs.

Editor note:	How to handle different AMF and LMF case is FFS.

4.	The GMLC invokes the Namf_Location_ProvideRanging_Sidelink positioningInfo Rquest service operation towards the AMF to request the Ranging/Sidelink Positioning information of the 2 UEs.

5.	If the UE is in CM-IDLE, the AMF initiates a network triggered Service Request procedure as defined in clause 4.2.3.3 of TS 23.502 [9] to establish a signalling connection with the UE.

6.	If there is privacy check requirement, the UE must either be notified or notified with privacy verification. The 2 UEs notify its user the Ranging/Sidelink Positioning request and provide the checked result to the AMF.

7. If granted in step6, the AMF perform the LMF selection with supporting Ranging/Sidelink Positioning capable consideration.

8.	The AMF invokes the Nlmf_Location_Determine Ranging/Sidelink Positioning service operation towards the LMF to request the Ranging/Sidelink Positioning information of the 2 UEs.

9.	After receiving the AMF request in step 8, the LMF performs the NW assistant Ranging/Sidelink Positioning with the 2 UEs with following possible aspects:

-	LMF obtains the Ranging/Sidelink Positioning capability of the 2 UEs, similar to the Procedures related to capability transfer in clause 5.1 of TS 37.355 [10];

-	LMF determines the role of the 2 UEs; which one takes the role of Reference UE, which one takes the role of Target UE.

-	LMF obtain the Ranging/Sidelink Positioning location information, similar to the Procedures related to Location Information Transfer in clause 5.3 of TS 37.355 [10];

-	LMF calculates the Ranging/Sidelink Positioning result between the 2 UEs.

10.	After performing the NW assistant Ranging/Sidelink Positioning procedure, the LMF returns the Nlmf_Location_Determine Ranging/Sidelink Positioning response towards the AMF to return Ranging/Sidelink Positioning result between the 2 UEs.

11.	The AMF returns the Namf_Location_ProvideRanging_Sidelink positioningInfo Reponse towards the GMLC return Ranging/Sidelink Positioning result between the 2 UEs.

12.	The (H)GMLC sends the location service response to the AF (via the NEF) with the Ranging/Sidelink Positioning result between the 2 UEs.

### 6.23.3	Impacts on Existing Nodes and Functionality

The solution has impacts on the following entities:

LMF:

-	Support NW assistant Ranging/Sidelink Positioning procedure.

AMF:

-	LMF selection with NW assistant Ranging/Sidelink Positioning support.

GLMC:

-	support check the UE privacy profile of supporting Ranging/Sidelink Positioning.

NEF:

-	support Ranging/Sidelink Positioning service exposure.

UDM:

-	support UE privacy profile of supporting Ranging/Sidelink Positioning subscription.

UE:

-	Supports the Ranging/SL positioning information negotiation with LMF.

## 6.24	Solution #24: Service initiated by Application server over user plane

### 6.24.1	General

This solution is to address KI#7.

The communication between application server and one of the UEs subject to Ranging/SL positioning for transmitting the service request and result reporting can be over user plane when the service request is initiated by the application server and there's already an existing PDU session between application server and one of the UEs subject to Ranging/SL positioning.

This saves the efforts for retrieving AMF, selecting LMF and the required signalling (e.g. NAS, signalling between UE and LMF if LMF has to be involved).

### 6.24.2	Functional descriptions

When the service request is initiated by the application server and there's already an existing PDU session between application server and one of the UEs subject to Ranging/SL positioning, the application server sends the Ranging/SL positioning service request to the UE and receives the result from the UE over application layer.

### 6.24.3	Procedures

Figure 6.24.3-1 Procedure of Service initiated by Application server over user plane

1.	Service authorization policy/parameters are provisioned to UE1 and UE2.

2.	If there is already a PDU session between application server and UE1, the Ranging/SL positioning service request is sent to UE1 at application layer, which includes Reference UE ID, Target UE ID, Result content (distance, angle or both) and Required QoS, etc.

3.	UE1 initiates Ranging/SL positioning procedure to UE2 to trigger the measurement and the calculation of the result. The Ranging/SL positioning procedure includes Ranging/Sidelink positioning discovery and service operations.

4.	UE1 sends the Ranging/SL positioning result to the AF over application layer.

Editor's note:	It is FFS how privacy checking and authorization is performed.

### 6.24.4	Impacts on services, entities, and interfaces

The solution has impacts in the following entities and interfaces:

-	UE: supports initiating Ranging/SL Positioning procedure.

## 6.25	Solution #25: Ranging/SL positioning service initiated by SL Positioning Client UE

### 6.25.1	General

This solution is to address KI#6.

A Ranging/SL positioning service request can be initiated by a UE, which may be one of the 2 UEs subject to Ranging/SL positioning (i.e. UE1 and UE2) or a SL Positioning Client UE.

The service request may be initiated by the upper layer of one of the UEs subject to Ranging/SL positioning (i.e. UE1 and UE2) or may be transmitted from the SL Positioning Client UE to one of the UEs.

The SL Positioning Client UE can be any UE, which does not have to support Ranging/SL positioning. It may be in proximity or not in proximity of UE1/UE2. For the latter case, the SL Positioning Client UE has to transmit the service request over 5GS network to UE1/UE2, which assumes that both the SL Positioning Client UE and UE1/UE2 are in 5G coverage.

### 6.25.2	Functional descriptions

When the service request is initiated by the upper layer of UE1/UE2, the UE1/UE2 triggers the Ranging/SL positioning procedure to the other UE directly.

NOTE 1:	Ranging/SL positioning procedure includes Ranging/Sidelink positioning discovery and service operations, which will be addressed in the solutions under KI#3 and KI#4.

When the service request is initiated by the SL Positioning Client UE, the communication between the SL Positioning Client UE and UE1/UE2 for transmitting the service request and result reporting can be over PC5 or Uu:

-	If the third UE is in proximity of UE1/UE2, the service request and the result reporting are transmitted over PC5;

-	Otherwise, the service request and the result reporting are transmitted over Uu, i.e. through the network.

The SL Positioning Client UE attempts to discover UE1/UE2:

-	If either UE1 or UE2 is successfully discovered, the SL Positioning Client UE sends the service request to the discovered UE over PC5;

-	Otherwise, the SL Positioning Client UE sends the service request to one of the UEs over network.

If the SL Positioning Client UE sends the service request to UE1/UE2 over network, the SL Positioning Client UE sends the request to its serving AMF, and the serving AMF of the SL Positioning Client UE needs to select the UE under network coverage to deliver the service request. The serving AMF of the SL Positioning Client UE retrieves the serving AMF of UE1/UE2 from the UDM, and then sends the service request to the selected UE through its serving AMF over NAS. The selected UE triggers the Ranging/Sidelink positioning procedure.

NOTE 2:	The service request will only be sent to one of UE1 or UE2. If both UE1 and UE2 are in network coverage, i.e. serving AMF address(es) for both UE1 and UE2 can be retrieved from UDM, serving AMF of the SL Positioning Client UE randomly selects a UE to deliver the service request.

NOTE 3:	This solution assumes that UE1 and UE2 are capable of handling Ranging/Sidelink positioning measurement and result calculation, and no LMF involvement is needed.

The Ranging/SL positioning service request includes the following parameters:

-	Application Layer ID of Reference UE.

-	Application Layer ID of Target UE.

-	Result content: distance, angle or both.

-	Required QoS.

Editor's note:	Whether there are any other parameters in the Ranging/SL positioning service request is FFS.

GMLC maintains a mapping table between Application Layer ID(s) and SUPI of the UE. This can be pre-configured by the operator, or is registered by the UE during UE Policy Association Establishment. For the latter case, during the registration, UE includes Ranging/SL Positioning Application Layer ID(s) in the UE Policy Container to the PCF. If the Application Layer ID(s) is received, PCF registers it together with UE SUPI into the GMLC for later use.

For the case where the SL Positioning Client UE has to send the Ranging/Sidelink positioning request to the target UE/reference UE via 5GC, when AMF receives a Ranging/Sidelink positioning request from the SL Positioning Client UE, it queries GMLC for UE SUPIs based on their Application Layer IDs contained in the Ranging/Sidelink positioning request, and then continues with the following steps.

Editor's note:	It is FFS whether it is possible that the mapping table between Application Layer ID(s) and SUPI of the UE can be managed by other 5GC NF.

### 6.25.3	Procedures

#### 6.25.3.1	PC5 based procedure for Ranging/SL positioning initiated by the SL Positioning Client UE

This procedure applies to the use cases where the SL Positioning Client UE is in proximity of UE1/UE2.

This procedure assumes that UE1 and UE2 are capable of handling Ranging/Sidelink positioning measurement and result calculation.

In this procedure, the SL Positioning Client UE, initiates the service request for Ranging/SL positioning between UE1 and UE2.

If the SL Positioning Client UE and UE1 are the same UE, i.e. the service request is initiated from upper layer of UE1, then only step 1 and step 4 are needed.

6.25.3.1-1: PC5 based procedure for Ranging/SL positioning initiated by SL Positioning Client UE

1.	Service authorization policy/parameters are provisioned to the UE1 and UE2.

2.	The SL Positioning Client UE initiates Ranging/SL positioning device discovery procedure to discover either UE1 or UE2, and one of them is successfully discovered, e.g. UE1. This step is skipped in the following scenarios:

-	There is existing communication between the SL Positioning Client UE and UE1.

-	The SL Positioning Client UE and UE1 are the same UE, i.e. the service request is initiated from upper layer of UE1.

NOTE 1:	It is also possible that UE2 is discovered. In this solution, UE1 represents the UE that the SL Positioning Client UE decides to send the service request to.

NOTE 2:	Device discovery follows the Device discover procedure of V2X or ProSe or the Ranging/SL positioning Device discovery procedure.

3.	The SL Positioning Client UE sends the Ranging/SL positioning service request to UE1, including Application Layer ID of UE1 and UE2, Result content (distance, angle or both) and Required QoS, etc.

4.	UE1 initiates Ranging/SL positioning procedure to UE2 to trigger the measurement and the calculation of the result. Application Layer ID of UE1 and UE2 are used for the communication between UE1 and UE2 over PC5.

NOTE 3:	The Ranging/SL positioning procedure includes Ranging/Sidelink positioning discovery and service operations. Whether DDNMF is involved is determined by the solution developed under KI#3.

5.	UE1 sends the Ranging/SL positioning result of UE1 and UE2 to the SL Positioning Client UE in the response.

#### 6.25.3.2	Network based procedure for Ranging/SL positioning initiated by SL Positioning Client UE (without involving LMF)

In this procedure, the SL Positioning Client UE initiates the service request for Ranging/SL positioning between UE1 and UE2.

This procedure applies to the use case where the SL Positioning Client UE is not in proximity of either UE1 or UE2 and both of the SL Positioning Client UE and UE1/UE2 are under network coverage.

This procedure assumes that UE1 and UE2 are capable of handling Ranging/Sidelink positioning measurement and result calculation.

6.25.3.2-1: Network based procedure for Ranging/SL positioning initiated by SL Positioning Client UE (without LMF involvement)

1.	Service authorization policy/parameters are provisioned to the UE1 and UE2.

2.	The SL Positioning Client UE failed in discovering UE1 and UE2 over PC5. The SL Positioning Client UE sends the Ranging/SL positioning service request to its serving AMF, i.e. AMF3, including Application Layer ID of UE1 and UE2, Result content (distance, angle or both) and Required QoS, etc.

3.	AMF3 selects a GMLC based on local configuration or by querying the NRF to get GMLC address. AMF3 queries GMLC for SUPI of UE1 and UE2 based on their Application Layer IDs.

Editor's note:	It is FFS how AMF3 selects the right GMLC(s) managing Application Layer ID and SUPI mapping of UE1&UE2, if multiple GMLCs are deployed.

4.	AMF3 checks UDM to acquire the serving AMF of UE1/UE2. If serving AMF address of UE1, i.e. AMF1, is retrieved, AMF3 sends the Ranging/SL positioning service request to AMF1, including SUPI value of UE1&UE2 as well as their Application Layer IDs.

NOTE 1:	It is also possible that serving address of UE2, is retrieved. In this solution, AMF1 represents the AMF that AMF3 decides to send the service request to.

5.	AMF1 checks the privacy of UE1 and UE2 based on their SUPIs to see if the SL Positioning Client UE is allowed to have the Ranging/SL positioning information of UE1 and UE2.

Editor's note:	How AMF1 performs service authorization and privacy checking will be developed by SA3.

6.	AMF1 sends the Ranging/SL positioning service request to UE1 over NAS, including UE1 and UE2's Application Layer IDs.

7.	UE1 initiates Ranging/SL positioning procedure to UE2 to trigger the measurement and the calculation of the result.

NOTE 2:	The Ranging/SL positioning procedure includes Ranging/Sidelink positioning discovery and service operations. Whether DDNMF is involved is determined by the solution developed under KI#3.

8.	UE1 sends the Ranging/SL positioning result to the AMF1 in the response.

9.	AMF1 forwards the Ranging/SL positioning result to the AMF3 in the response.

10.	AMF3 forwards the Ranging/SL positioning result to SL Positioning Client UE in the response.

#### 6.25.3.3	Ranging/SL positioning Application Layer ID registration

6.25.3.3-1: Ranging/SL positioning Application Layer ID registration

1.	PCF receives from UE the list of Ranging/SL Positioning Application Layer ID(s) corresponding to the application(s) in the UE Policy Container contained in the Npcf_UEPolicyControl Create Request during UE Policy Association Establishment.

Editor's note:	Whether there is security issue, e.g. A malicious UE may hijack any other UE's service by registering other UE's application layer ID, will be evaluated and studied by SA WG3.

Editor's note:	How UE obtains the list of Ranging/SL Positioning Application Layer ID(s) corresponding to the application(s) during and after UE Policy Association Establishment is FFS.

2.	PCF registers the UE SUPI and the list of Ranging/SL Positioning Application Layer ID(s) relationship into GMLC.

3.	GMLC stores the UE SUPI and Ranging/SL Positioning Application Layer ID relationship, and responds PCF.

### 6.25.4	Impacts on services, entities, and interfaces

The solution has impacts in the following entities and interfaces:

-	UE: supports initiating Ranging/SL Positioning procedure; receives Ranging/SL Positioning service request from AMF or the third UE; sends Ranging/SL Positioning result to AMF or the 3rd UE.

-	AMF: receives and forwards the Ranging/SL Positioning service request to the UE1; receives and forwards the Ranging/SL Positioning result to the SL Positioning Client UE; SUPI query

-	GMLC: SUPI query and Application Layer ID registration

-	PCF: Application Layer ID registration

-	NAS: supports transmission of Ranging/SL Positioning service request and Ranging/SL Positioning result.

-	PC5: supports transmission of Ranging/SL Positioning service request and Ranging/SL Positioning result.

## 6.26	Solution #26: Functional Split and Positioning Protocol for SL Positioning

Editor's note:	This clause describes a solution addressing one or more key issues identified in clause 5. The structure of the clauses can be adjusted. The list of key issues which this solution attempts to resolve should be clearly indicated.

### 6.26.1	General

This solution proposes a functional split and positioning protocol to enable the support of Sidelink (SL) positioning (and Ranging). It is relevant to: KI#2, KI#3, KI#4, KI#5, KI#6, KI#7. This solution is proposed in the form of architectural requirements to implement the proposed functional split and positioning protocol.

In the following the term "Location Server" is used either:

-	As a generic term, therefore applicable to any of the following qualified terms; or

-	As a qualified term:

-	SL Positioning Server UE: residing in the UE

-	Location Server Platform: referring to a SUPL Location Platform (SLP) i.e. accessible via user plane

-	Location Server NF: residing in the network and accessible via control plane signalling i.e. LMF

In 5GS, the Location Server, whether in the user-plane (Platform) or in the control-plane (NF) is a "network" entity that does not reside in a UE. With SL positioning and Ranging aiming at enabling peer-to-peer operation (i.e. between UEs) without necessarily having any network involvement, the current architecture is not sufficient.

This solution therefore proposes that

-	the functionality offered towards the UE by a Location Server can also reside in a UE able to host such functionality as not all UEs supporting SL positioning may be able to position themselves; and

-	a Positioning Protocol be defined that can operate;

-	between a Target UE and a SL Positioning Server UE (if a Target UE needs assistance from a SL Positioning Server UE);

-	between a SL Positioning Server UE and a Reference UE, so the SL Positioning Server UE can gain additional reference information to help position a Target UE (similar to a Location Server NF getting information from base stations);

-	between a Target UE and a Reference UE, at least for the case when a Target UE need not rely on a SL Positioning Server UE to position itself.

It is also important to consider that SL Positioning or Ranging can operate with a Location Server Platform/NF. Synergies must therefore be sought between peer-to-peer and "network-based" (i.e. using a Location Server Platform/NF) SL positioning operations in terms of the related Positioning Protocol.

### 6.26.2	Functional Split

The following architectural requirements are proposed:

-	A UE supporting SL positioning may support functionality to act as a SL Positioning Server UE, for positioning of Target UE(s).

-	A Target UE supporting SL positioning may require assistance from a Location Server.

-	A Target UE supporting SL positioning may require reference information from Reference UE(s).

-	A SL Positioning Server UE may require reference information from Reference UE(s).

### 6.26.3	Positioning Protocol

-	A Sidelink Positioning Protocol (hereafter "RSPP" - the term is introduced in Solution #4) shall be defined to support SL Positioning.

-	A UE supporting SL Positioning shall support RSPP:

-	this UE may be a Target UE, a Reference UE or a SL Positioning Server UE.

-	A Location Server supporting SL Positioning shall support RSPP.

-	RSPP shall be able to operate between a Target UE and a Location Server, whether the Location Server is hosted by a UE or a network entity (i.e. NF or Platform).

-	RSPP shall be able to operate between a Target UE or SL Positioning Server UE, and a Reference UE (see note 1).

NOTE 1:	Operation with a Reference UE requires that RSPP also incorporate functions analogous to those offered today by the NRPPa protocol for the delivery of Reference UE configurations/measurements to the SL Positioning Server UE, and by the RRC protocol for the control of reference signal transmission/measurements between the Target UE and the Reference UE. Details are under RAN WG responsibility.

NOTE 2:	RSPP should be defined as a "standalone" extension of LPP i.e. RSPP can be supported (esp. in a SL Positioning Server UE) without having to support the entire LPP, while Location Servers (NF/Platform) already supporting LPP can be upgraded with the RSPP extension. The definition of RSPP is under RAN WG responsibility.

-	RSPP shall be able to operate securely over the User-Plane between a Target UE and a Location Server Platform.

-	RSPP shall be able to operate over the Control-Plane between a Target UE and a Location Server NF (LMF).

-	RSPP operating between two endpoint UEs shall operate over PC5-S between these endpoints i.e.:

-	between a Target UE and a SL Positioning Server UE;

-	between a Target UE or a SL Positioning Server UE and a Reference UE.

Editor's note:	PC5-U could be considered instead of PC5-S.

-	Should a Relay UE be necessary for a Target UE to communicate with a Location Server, the Relay UE shall not be required to support SL positioning or RSPP.

### 6.26.4	Impacts on services, entities, and interfaces

UE:

-	Support of RSPP.

-	Optional support of Location Server functionality i.e. SL Positioning Server UE.

-	When operating as a Target UE:

-	ability to require assistance from a Location Server if necessary;

-	ability to require reference information from Reference UE(s).

-	When operating as a SL Positioning Server UE:

-	ability to receive measurement data, location information from a Target UE;

-	ability to require reference information from Reference UE(s);

-	ability to provide assistance data to a Target UE.

-	When operating as a Reference UE:

-	ability to provide reference information to a SL Positioning Server UE or a Target UE upon request from these UEs.

LMF:

-	Support of RSPP.

-	Ability to receive measurement data, location information from a Target UE.

-	Ability to provide assistance data to a Target UE.

SUPL SLP:

-	Support of RSPP.

-	Ability to receive measurement data, location information from a Target UE.

-	Ability to provide assistance data to a Target UE).

NOTE:	RSPP, measurement data, reference information and assistance data are for RAN WGs to define.

## 6.27	Solution #27: LMF selection for Ranging/Sidelink positioning

### 6.27.1	General

This solution is to address KI#2, KI#3, KI#4, KI#5, KI#6, and KI#7, where LMF is involved for Ranging/SL positioning.

In some Ranging based service and sidelink positioning exposure solutions and network assisted sidelink positioning solutions, LMF is used for initiating the Ranging/SL positioning service request to the UE. LMF is originally introduced for the determination of UE location, however, not every LMF is capable of handling Ranging/SL positioning request, i.e. initiating Ranging/SL positioning procedure to the UE based on the request received from the AMF, generating Ranging/SL positioning result based on the measurement data and reporting the result to the AMF. If LMF is determined to be used, the AMF has to select a LMF supporting Ranging/SL positioning functionalities.

LMF may be deployed to support both Uu based positioning and Ranging/Sidelink positioning. Meanwhile, it is also possible that Ranging/Sidelink positioning dedicated LMF is deployed separately from the LMF used for Uu positioning, i.e. an LMF is assumed to support capabilities of Uu based positioning only, capabilities of Ranging/SL positioning only or both.

### 6.27.2	Functional descriptions

In Ranging/SL positioning service, LMF selection is performed at AMF when a Ranging/SL positioning request is received and the AMF determines to use the LMF for ranging/SL positioning.

To support Ranging/SL positioning service, Ranging/SL positioning capabilities are required at LMF, which include:

-	Triggering Ranging/SL positioning between two UEs or between one UE and multiple UEs based on the request received from the AMF;

-	Ranging/SL positioning result calculation based on Ranging/SL positioning measurement data, which include:

-	Calculation of the distance between 2 UEs.

-	Calculation of the direction of one UE from the other UE.

-	Calculation of high accuracy distance between 2 UEs.

-	Calculation of high accuracy direction of one UE from the other UE.

-	Calculation of the distance between one UE and each one of the multiple UEs.

-	Calculation of the direction of one UE from each one of the multiple UEs.

-	Calculation of high accuracy distance between one UE and each one of the multiple UEs.

-	Calculation of high accuracy direction from one UE to each one of the multiple UEs.

-	Ranging/SL positioning result reporting.

The above-mentioned Ranging/SL positioning capability factors should be taken into consideration when selecting an LMF for Ranging/SL positioning.

Furthermore, Requested Quality of Service information needs to include the Ranging/SL positioning Quality of Service information, i.e. distance accuracy and direction accuracy.

Additionally, AMF also needs to consider the requested Ranging/SL positioning content when selecting LMF, i.e. distance, direction or both.

Editor's note:	Whether more factors need to be considered for LMF selection for Ranging/SL positioning is FFS.

When LMF is selected to perform network assisted sidelink positioning, AMF needs to select a LMF capable of network assisted sidelink positioning, i.e. by considering both factors of LMF selection for Uu based positioning, as defined in clause 5.1 of TS 23.273 [11] and factors of LMF selection for Ranging/SL positioning.

### 6.27.3	Impacts on services, entities, and interfaces

The solution has impacts in the following entities and interfaces:

-	AMF: selects LMF based on service request and new factors.

## 6.28	Solution #28: Using LMF for Ranging/Sidelink positioning

### 6.28.1	General

This solution is to address KI#2, KI#3, KI#4, KI#5, KI#6, and KI#7, where LMF is involved for Ranging/SL positioning.

The overall procedure of Ranging/Sidelink positioning consists of the following steps:

-	Ranging/Sidelink positioning request (e.g. from UE, AMF, LMF).

-	Measurement between 2 UEs or between one UE and multiple UEs.

-	Result calculation (e.g. at UE or at LMF, or at both).

-	Ranging/Sidelink positioning response (e.g. to UE, AMF, LMF).

If LMF is involved in the procedure and if Ranging/Sidelink positioning result calculation is supported by LMF, result calculation can be tasked to LMF. This is beneficial to the UE with reduced capability, e.g. who doesn't support Ranging/Sidelink positioning result calculation, or who wants to save the power consumed for result calculation. It is especially useful for the case of network assisted sidelink positioning to facilitate positioning of a UE in coverage or with partial coverage when Uu positioning is not possible and when a higher location accuracy is required.

In the meantime, it is also possible that both UE and LMF will be involved for result calculation, e.g. UE provides a rough Ranging/Sidelink positioning result with a simple calculation, which can be used by the LMF together with other inputs to derive a high accuracy result.

Since the result calculation can be performed at UE or LMF or both, they have to be coordinated, so that both UE and LMF understand its role in the procedure.

### 6.28.2	Functional descriptions

In terms of which node to perform Ranging/Sidelink positioning result calculation, 3 modes are defined:

-	UE based Ranging/Sidelink positioning result calculation: UE performs Ranging/Sidelink positioning result calculation.

-	LMF based Ranging/Sidelink positioning result calculation: LMF performs Ranging/Sidelink positioning result calculation.

-	Hybrid: UE and LMF jointly perform Ranging/Sidelink positioning result calculation.

LMF determines the Ranging/Sidelink positioning result calculation mode based on:

-	LMF Ranging/Sidelink positioning capability.

-	UE Ranging/Sidelink positioning capability.

-	UE preference of Ranging/Sidelink positioning calculation node: i.e. UE, LMF or both.

-	Operator policy, i.e. operator preference of UE, LMF or both.

If Ranging/Sidelink positioning result calculation is only supported by UE, UE based mode is selected; if Ranging/Sidelink positioning result calculation is only supported by LMF, LMF based mode is selected; if Ranging/Sidelink positioning result calculation is supported by both UE&LMF, LMF determines the node based on UE preference and operator policy.

After Ranging/Sidelink positioning result calculation mode is determined, the LMF includes in the Ranging/Sidelink positioning service request the requested info (i.e. measurement data or Ranging/Sidelink positioning result or both).

Requested info is determined by the selected mode, which may include:

-	Measurement data if LMF based mode is selected;

-	Ranging/Sidelink positioning result if UE based mode is selected;

-	Both Measurement data and Ranging/Sidelink positioning result if hybrid mode is selected

During Ranging/Sidelink positioning procedure, UE performs:

-	measurement only if only measurement data is requested;

-	both measurement and calculation if Ranging/Sidelink positioning result is requested.

After performing Ranging/Sidelink positioning, UE1 reports based on the requested info.

### 6.28.3	Procedures

#### 6.28.3.1	Procedure of LMF triggered Ranging/sidelink positioning

This procedure applies to the following cases as a part of the overall procedure when Ranging/Sidelink positioning between 2 UEs is triggered by LMF:

-	Ranging/Sidelink positioning with the assistance of assistant UE.

-	Network assisted Sidelink Positioning for In Network Coverage and Partial Network Coverage.

-	Ranging and sidelink positioning service exposure to a SL Positioning Client UE.

-	Ranging and sidelink positioning service exposure to AF or 5GC NF.

It is used to determine whether the Ranging/Sidelink positioning result or Ranging/Sidelink positioning measurement data should be provided to the LMF.

Figure 6.28.3.1-1: Procedure of LMF triggered Ranging/sidelink positioning

1.	AMF receives a Ranging/Sidelink positioning request from a UE or a GMLC or any 5GC NF.

2.	AMF selects a serving LMF for UE1.

3.	AMF sends the Ranging/Sidelink positioning request to the selected LMF.

4.	LMF performs Ranging/Sidelink positioning capability negotiation with UE1, see clause 6.28.3.2 for details of LMF triggered Ranging/Sidelink positioning capability negotiation.

5.	LMF determines the Ranging/Sidelink positioning result calculation mode and the requested info based on LMF Ranging/Sidelink positioning capability, UE Ranging/Sidelink positioning capability, UE preference of Ranging/Sidelink positioning calculation node: i.e. UE, LMF or both and Operator policy, i.e. operator preference of UE, LMF or both, as described in clause 6.28.2.

6.	LMF sends the Ranging/Sidelink positioning request to UE1, including the requested info (i.e. measurement data or Ranging/Sidelink positioning result or both).

7.	UE1 performs Ranging/Sidelink positioning with UE2. If only measurement data is requested or LMF based mode is selected, UE does not have to take the result calculation.

NOTE	Details of the steps are developed in the solutions of KI#4.

8a.	UE1 reports Ranging/Sidelink positioning result to LMF if it is requested.

8b.1	UE1 reports Ranging/Sidelink positioning measurement data to LMF if it is requested.

8b.2	LMF performs Ranging/Sidelink positioning result calculation based on the measurement data received from UE1.

9.	LMF reports the Ranging/Sidelink positioning result to the AMF.

10.	AMF reports the Ranging/Sidelink positioning result.

#### 6.28.3.2	Procedure of LMF initiated Ranging/sidelink positioning capability negotiation between UE and LMF

Figure 6.28.3.2-1: Procedure of LMF initiated Ranging/sidelink positioning capability negotiation between UE and LMF

1.	LMF sends the Ranging/Sidelink positioning capability request to the UE1, which includes LMF Ranging/Sidelink positioning capability, UE1 ID and UE2 ID.

2.	If there is no available UE2 Ranging/Sidelink positioning capability for result calculation, UE1 triggers Ranging/Sidelink positioning capability negotiation procedure to acquire UE2 Ranging/Sidelink positioning capability for result calculation.

Editor's note:	The details on how Ranging/Sidelink positioning capability is negotiated between UE1 and UE2 are FFS.

3.	UE1 responds to LMF with UE1/UE2 Ranging/Sidelink positioning capability. Optionally, UE1 may also include in the response UE1 preference of Ranging/Sidelink positioning result calculation mode, which is determined based on LMF Ranging/Sidelink positioning capability and UE1/UE2 Ranging/Sidelink positioning capability.

### 6.28.4	Impacts on services, entities, and interfaces

The solution has impacts in the following entities and interfaces:

-	UE: supports Ranging/Sidelink positioning capability negotiation between UEs and between LMF and UE; determines its role in the Ranging/Sidelink positioning procedure based on the requested info.

-	LMF: supports Ranging/Sidelink positioning capability negotiation between LMF and UE; determines result calculation mode.

Editor's note:	The interactions between the UE and LMF regarding the capability negotiation and switching of the mode should be determined by RAN WGs, and alignment is needed once there's any conclusion in RAN.

## 6.29	Solution #29: Service continuity of periodic ranging with assistant UE

### 6.29.1	General

This solution address the key issue#2 on "Ranging service operation procedure with the assistance of another UE". Specifically, this solution focuses on maintaining the service continuity of periodic ranging with Assistant UEs.

### 6.29.2	Functional descriptions

There are scenarios where periodic ranging operation between two UEs is required, e.g. tracking the relative position of two moving cars on the highway at a certain interval, and there is another car in-between the two cars.

The scenario is illustrated below, in which two assumptions are made:

1.	the pair of UE (UE 1 and UE 2) are moving towards the same direction

2.	assistant UE is fixed, and available during the periodical ranging period, with the proper deployment e.g. RSU.

Figure 6.29.2-1

In these scenarios, if the direct ranging/sidelink positioning between the target UE and the reference UE cannot be supported, or if the ranging/sidelink positioning is not sufficiently accurate, the assistance of another UE (e.g. assistant UE, deployed as RSU) is needed, and direct communication between the assistant UE and the target UE/reference UE should be supported. During the periodic ranging operation with assisted UE, when the distance between the assistant UE and the target UE and/or the distance between the assistant UE and the reference UE increases to an extent that the assistant UE cannot continue the assistance, the ranging service operation might go through a disruption. This solution is proposed to solve the problem of maintaining service continuity of periodic ranging with assistant UE.

During each cycle of the periodic ranging process, after obtaining the ranging measurements, UE1 immediately performs a ranging service check according to all the measurement data collected so far to determine whether the current assistant UE can continue the assistance in the next cycle, and if not, UE1 would restart the device discovery procedure to find another assistant UE. In this way, disruptions of ranging service is avoided.

Functions of the Target UE:

-	Ranging/sidelink positioning measurements.

-	Determine whether the current assistant UE can continue to assist ranging in the next cycle according to the previous measurements.

-	Ranging result calculation.

The Target UE (UE1) performs ranging service check according to all the previous measurements (e.g. the distances, the received ranging signal power, and the signal directions) and the tendencies that can be inferred or calculated from the measurements (e.g. the moving direction and velocity of the target UE). Thresholds of the relevant measurements and parameters can be pre-set for a quick determination.

Editor's note:	An alternative solution is that the determination on the use of assistant UE and which assistant UE is selected are independent for each cycle of periodic Ranging. When and how to determinate whether to use the current assistant UE, discover a new assistant UE or no assistant UE in different cycles of periodic Ranging is FFS.

### 6.29.3	Procedures

Figure 6.29.3-1: High-level procedure for ranging operation with assistant UE

1.	UE1 and UE2 get the ranging authorization policy from PCF during the registration procedure. The ranging authorization policy may include whether the UE is authorized to perform ranging operation with assistant UE. Assistant UE1 and Assistant UE2 get the ranging authorization policy from PCF during the registration procedure. The ranging authorization policy may include whether the UE is authorized to act as an assistant UE for ranging operation.

2.	When the UE1 gets the ranging request from the application layer, but the direct ranging operation between the UE1 and UE2 cannot be supported, UE1 will perform assistant UE discovery procedure.

3.	Ranging and sidelink positioning is performed among UE1, UE2 and the discovered assistant UE (Assistant UE1), and the ranging result is calculated. For detailed procedures of this step, Sol#16 can be a reference.

4.	After the ranging/sidelink positioning measurement, UE1 performs ranging service check to determine whether it can continue to provide assistance in the next cycle, according to previous ranging measurements including the distances from UE1 and UE2, the received ranging signal power, the moving direction of UE1 and UE2; thresholds of these measurements maybe pre-set.

NOTE 1:	For cases e.g. UE(cars) are on the barricaded highway or make a U-turn, the on-going periodic ranging session can be terminated by the UE.

5.	When UE1 determines that the Assistant UE1 cannot continue to assist the ranging service in the next cycle, it performs another assistant UE discovery procedure to discover Assistant UE2.

NOTE 2:	Assistant UE 2,3, N can be assumed to be discovered during the lifetime of the periodic ranging service between UE1 and UE2. For example, in V2X scenario multiple RSUs supporting the assistant UE functionality are deployed at the roadside of high-way.

6.	Ranging and sidelink positioning is then performed among UE1, UE2 and the newly discovered assistant UE (Assistant UE2), and the ranging result is calculated.

7.	UE1 performs ranging service check to determine whether the current assistant UE can continue to provide assistance in the next cycle.

### 6.29.4	Impacts on services, entities, and interfaces

The solution has impacts on the following entities:

Same impacts as proposed in solution#16, and in addition:

Target UE:

-	Perform "ranging service check" according to the previous measurements

## 6.30	Solution #30: Ranging service operation with the assistant UE(s)

### 6.30.1	Introduction

The solution provides methods for ranging service operation with the assistant UE.

### 6.30.2	Functional Description

In this solution, it assumes UE1 and UE2 have established communication path by existing technologies, e.g. direct communication via PC5, communication via ProSe UE-to-UE relay.

The principles in the ranging operation with the assistant UE are as follows:

-	UE1 and UE2 discovers assistant UEs by any solutions for KI#3, e.g. Model A (assistant UEs Announcement), Model B (Solicitation/Response).

-	UE1 and UE2 negotiate for assistant UEs selection

-	UE1 and UE2 perform ranging service with selected assistant UEs independently and shares the ranging result to complete ranging calculation.

NOTE 1:	Selected assistant UE(s) may be and may not be on the communication path between UE1 and UE2. For example, when UE1 communicates with UE2 via a ProSe UE-to-UE relay, UE1 and UE 2 select another UE as assistant UE rather than the UE-to-UE relay.

NOTE 2:	In this solution, any Target UE/ Reference UE defined in solutions for KI#3 and KI#4 can act as assistant UE without additional enhancement.

### 6.30.3	Procedures

Figure 6.30.3-1: Procedure of Ranging/Sidelink positioning with assistant UEs

0a.	UE1, UE2 and assistant UE are authorized for Ranging/Sidelink positioning service. The details will be defined by solutions for KI#1.

0b.	Communication path between UE1 and UE2 is established. How to establish communication path is based on existing technologies, e.g. direct PC5 communication, communication via UE-to-UE relay.

1.	Once UE1 decides to trigger Ranging/Sidelink positioning with UE2 via assistant UEs, UE1 performs Ranging/Sidelink Positioning device discovery to discover available assistant UEs. Ranging/Sidelink Positioning device discovery may use any discovery solutions for KI#3. UE1 may select assistant UE(s) based on discovered assistant UEs' capability.

2.	UE1 sends Ranging/Sidelink positioning request to UE2 via established communication path as defined in solutions for KI#4. Additionally, the Ranging/Sidelink positioning request includes selected assistant UE list.

NOTE:	Ranging/Sidelink positioning request/response are sent via SR5 reference point defined in clause 4.3.2. Whether messages are carried by PC5 user plane or control plane will be aligned with RAN WGs.

3.	UE2 discovers available assistant UEs as step 2 and performs assistant UE selection based on discovered assistant UEs' capability.

4.	UE2 sends Ranging/Sidelink positioning response with selected assistant UEs.

5.	UE1 performs Ranging/Sidelink positioning with selected assistant UEs.

6.	UE2 performs Ranging/Sidelink positioning with selected assistant UEs.

7.	The Ranging/Sidelink positioning result with selected assistant UEs is shared between UE1 and UE2 for calculation of Ranging/Sidelink positioning result between UE1 and UE2.

### 6.30.4	Impacts on services, entities, and interfaces

UE:

-	support of Ranging/Sidelink positioning with assistant UEs.

Assistant UE:

-	a UE support Target UE/ Reference UE.

## 6.31	Solution #31: Enabling SL positioning for the UE out of coverage

### 6.31.1	General

This solution addresses the KI#5 with a way how to provide location service when the requested UE is out of coverage.

### 6.31.2	Functional Description

The overall procedure involves AMF, LMF, located UE(s) and a requested UE.

In this solution, in addition to existing functionality, following major functionality are added.

-	UE registered its capability for SL positioning and is configured with policies and parameters for SL positioning (including the case when UE is out of coverage)

-	AMF performs the selection of candidate list of located UEs when UE is determined not reachable and compose a security container including protected location request information for the requested UE.

-	LMF handles NW triggered SL positioning with located UEs and a requested UE.

-	Located UE sets up a PC5 connection with a requested UE and performs the SL positioning operation.

-	A requested UE validates a forwarded location request via a located UE.

This solution can support UE based SL positioning calculation, LMF based SL positioning calculation, and hybrid mode based on configuration in the LMF or negotiation between LMF and located UEs.

This solution supports both immediate location request and deferred location request with triggering condition.

The security container is protected and validated by security credentials shared between UE and the 5GCN.

In this solution, it is assumed that UE and AMF do not remove UE's context and AMF maintain UE's last known location information even though UE is considered as deregistered. If there is no context available for the UE in AMF, AMF will reject the received location request.

Editor's note:	The detail security mechanism is to be determined by SA WG3.

### 6.31.3	Procedures

#### 6.31.3.1	Procedure of immediate location request procedure with SL positioning

Figure 6.31.3.1-1: Procedure of immediate location request with SL positioning

0.	It is assumed that UE registered its capability for SL positioning and is provisioned with policies and parameters for SL positioning (including the case when UE is out of coverage). It is assumed that located UEs registered its capability for SL positioning and capability for located UE at AMF.

1.	AMF is triggered to initiate location request for UE1. When UE1 is determined unreachable, AMF compose list of located UEs based on last known UE1's location information. If there is no context available for the UE in AMF, AMF will reject the received location request.

Editor's note:	It is FFS whether existing timer can be reused or new timer needs to be define for determining how long the context needs to be maintained when UE is deregistered.

AMF selects a LMF and send a location request for UE1 to the selected LMF. In the location request, AMF includes a security container to protect the location request information to UE1. AMF may inform list of located UEs to LMF.

The security container includes information that indication of immediate or deferred location request, information for periodic-triggered location report when deferred location request e.g. type of deferred location request and any parameters for the deferred location request. signature for integrity protection.

Editor's note:	For UE deregistered from NW, how security container is composed and validated between AMF and UE1 is FFS and needs coordination with SA WG3.

2.	After receiving location request, the LMF sends NW triggered SL positioning Request via (R)AN to the selected located UEs which includes the security container information.

It is assumed that LMF is configured or has exchanged capability of Ranging/sidelink positioning of each located UEs before NW triggered SL positioning Request.

Editor's note:	How the located UE is selected by the LMF so that UE1 can be successfully discovered is FFS.

3.-4.	The located UE performs discovery procedure with UE1 and if UE1 is discovered, the located UE and UE1 perform security association and PC5 unicast connection setup procedure as mentioned in TS 23.304 [4]. When receiving discovery requests from multiple located UEs, UE1 may select one located UE and perform step 4 with the selected located UE.

5.	After PC5 unicast connection setup, the located UE sends SL positioning Request including the received security container.

6.	After validating the message with information in the security container, UE1 may accept the request with SL positioning Response.

7.	After receiving the SL positioning Response, the located UE may perform SL positioning with the target UE.

8.	After successfully finishing SL positioning procedure, the located UE responds to the LMF via ®AN using NW triggered SL positioning Response which includes the result of SL positioning with UE1. LMF may determine the location information of UE1 using information included in received SL positioning Response.

9.	LMF may response AMF with location information of UE1.

#### 6.31.3.2	Procedure of deferred location request procedure with SL positioning

Figure 6.31.3.2-1: Procedure of deferred location request with SL positioning

0.	It is assumed that UE registered its capability for SL positioning and is provisioned with policies and parameters for SL positioning (when UE is out of coverage). It is assumed that located UEs registered its capability for SL positioning and capability for located UE at AMF.

1.	AMF is triggered to initiate location request for target UE. When target UE is determined unreachable, AMF compose list of located UEs based on last known target UE's location information. If there is no context available for the UE in AMF, AMF will reject the received location request.

AMF selects LMF and send a location request for target UE to the selected LMF. In the location request, AMF includes a security container to protect the information to target UE. AMF may inform list of located UEs to LMF.

The security container may include a indication of immediate or deferred location request, information for periodic-triggered location report when deferred location request e.g. type of deferred location request and any parameters for the deferred location request. list of selected located UEs, and signature for integrity protection.

Editor's note:	For UE deregistered from NW, how security container is composed and validated between AMF and target UE is FFS and needs coordination with SA WG3.

2.	After receiving location request, the LMF sends NW triggered SL positioning Request v®(R)AN to the selected located UEs which includes the security container information.

3.-4.	The located UE performs discovery procedure with target UE and if target UE is discovered, the located UE and UE1 perform security association and PC5 unicast connection setup procedure as mentioned in TS 23.304 [4].

5.	After PC5 unicast connection setup, the located UE sends SL positioning Request including the received security container.

6.	After validating the message with information in the security container, the target UE may accept the request with SL positioning Response.

When the location type indicated in the security container is deferred location request, the target UE will store the information and when the trigger conditions are satisfied, the target UE may initiate MO-LR type location request.

7.	After receiving the SL positioning Response, the located UE_1 may respond LMF with NW triggered SL positioning Response to indicate the request was delivered well.

8.	After receiving SL positioning Response, the LMF may respond AMF with Location Response to indicate the Location Request was successfully delivered.

9.	When triggering condition was satisfied, the target UE may initiate procedure for 5GC-MO-LR request.

10.	The target UE performs discovery procedure to find available located UEs. When the target UE discover a located UE, the target UE may check whether the located UE_2 is in the list of located UEs which is received in the security container.

10.-11.	After finding the located UE_2 is in the list of located UEs, target UE and the located UE_2 perform security association and PC5 unicast connection setup procedure as mentioned in TS 23.304 [4].

12.	The target UE send a NW Assisted SL positioning Request to the located UE_2.

NW Assisted SL positioning Request includes the information required to compose 5GC-MO-LR Request message at the located UE_2 e.g. information about the consumer of the location information of target UE, i.e. AF ID or LCS client ID, QoS information for the location request, and requested positioning method.

13.	The target UE and the located UE_2 performs SL positioning.

14.	The located UE_2 sends 5GC-MO-LR request to LMF which include target UE ID, located UE ID, consumer AF or LCS client ID, result of SL positioning between located UE_2 and the target UE.

15.	After receiving 5GC-MO-LR request, LMF may perform procedure to determine location information of the located UE_2.

16.	After determining the location information of the target UE, LMF may send Location Report to the AMF which include target UE ID, location information of the target UE, and consumer AF or LCS_client ID.

### 6.31.4	Impacts on services, entities, and interfaces

The solution has impacts in the following entities and interfaces:

-	UE: policy provisioning for SL positioning and performing SL positioning with security update.

-	AMF: managing list of located UE for SL positioning of UE out of coverage and composing security container to target UE out of coverage.

-	LMF: location determination with SL positioning.

## 6.32	Solution #32: Involving Multiple Reference UEs for Sidelink positioning

### 6.32.1	Description

Involving multiple Reference UEs can improve the position accuracy and is sometimes essential to be able to calculate a position through triangulation/tilateration, e.g. if the Target UE is out of coverage.

The following deployment scenarios are being considered:

-	Multiple Reference UEs are part of a "fixed" installation, for example to cover a e.g. room, building or factory floor.

-	Multiple Reference UEs are close to a Target UE, but not necessarily have a fixed position or role.

An example of how this could look like is illustrated below. The Target UE is able to perform ranging with both Reference UE 1 and Reference UE 2. The results can be combined to estimate the Target UE's position in a relative coordinate system or possibly geographical coordinate system (if the geographical positions of the Reference UEs are known). By collecting the ranging measurements/results from the various UEs involved,  the sidelink position can be calculated more accurately than when only a single Reference UE would be used. Not only because the ranging measurements with multiple Reference UEs can be used in the calculation, but also e.g. if a Target UE can discover one reference UE of a "fixed" installation, but is not able to discover another reference UE that is part of the same installation, it tells something about the Target UE's location, since certain areas can be excluded from the location calculation. Also Reference UEs may be instructed to perform ranging between themselves to further improve the accuracy, in particular in case their position is not always fixed.

These multiple Reference UEs may form some kind of "constellation" similar to a satellite constellation for GNSS.

Figure 6.32.1-1: Illustration of a fixed installation of Reference UEs in a room or factory floor

In this solution, the above scenarios are considered for in-coverage, partial-coverage and out-of-coverage situations, whereby the LMF (or a subset implemented by a reference UE/SL Positioning Server UE) is considered to have the central coordinating role of the sidelink positioning involving multiple Reference UEs and is also responsible for determining the position of the Target UE. The case of a Target UE performing UE-based positioning and coordinating individual ranging sessions with multiple reference UEs is not described as part of this solution.

In this solution the term Ranging/Sidelink Positioning Protocol (RSPP) is used as defined in solution #26.

Below each of the coverage situations is described in more detail:

-	In-coverage: Both the Target UE and Reference UEs can connect to an LMF to coordinate and configure the ranging between the Target UE and the Reference UEs. The LMF may keep track of Reference UEs and in particular Located UEs in a certain area to be able to determine a group of Reference UEs that could serve a Target UE, or be provisioned by an external application e.g. through NEF with information about a "constellation" of Reference UEs in a certain area and their locations. If the location of the Reference UEs is not yet known, the LMF may initiate a location request with those Reference UEs.

If the position of the Target UE needs to be determined, the Target UE may send a location request to the LMF or vice-versa. After this, the Target UE initiates a ranging discovery procedure as described in the various solutions for KI#3, and initiate a ranging session with a discovered Reference UE by transmitting a Direct Connection Request. The Target UE or Reference UE can report information about the ranging session (such as session identifier, and identifiers of the Target UE and Reference UE) to the LMF. Based on the information about ranging constellations or other Reference UEs in the area, the LMF can invite other Reference UEs to join the ranging session (with the indicated identifier) or ask the Target UE or a Reference UE to do so, and in this way coordinate the joint ranging of a Target UE for improved location estimation.

In the figure below, the Target UE discovers and initiates a ranging session with Reference UE 1, after which Reference UE 2 is also invited to join the same ranging session.

Figure 6.32.1-2: High-level overview of in-coverage scenario

Once the ranging session with multiple Reference UEs is properly set up, the UEs involved can perform ranging to measure/calculate distances and/or angles between them, and report these to the LMF to estimate the location of the Target UE.

NOTE 1:	The details of these procedures will be defined by RAN WGs.

-	Partial coverage: In this situation, the Target UE is out-of-coverage, but at least one of the Reference UEs is in coverage and can connect to the LMF. In this situation, the Reference UEs can operate in a very similar way as the in-coverage case. The Target UE can initiate a ranging discovery procedure similarly as for the in-coverage case. The only difference is that the Target UE cannot contact the LMF directly to initiate a location request, receive configuration information or send its measurements. Instead, one or more the Reference UEs that are connected with LMF have to do this on behalf of the LMF. To this end, the Reference UE may implement part of the LMF's functionality or perform forwarding of messages between the Target UE and the LMF or a combination thereof.

In the figure below, the Target UE is out of coverage, and Reference UE 2 as well. Reference UE 1 will perform the communication with the LMF on behalf of the Target UE and Reference UE 2.

Figure 6.32.1-3: High-level overview of partial coverage scenario

-	Out-of-coverage: In this situation, the Target UE as well as the Reference UEs are out of coverage. To not deviate too much from the previous situations, a solution is for one or more of the Reference UEs to implement a subset of the LMF's functionality and support the same protocols (e.g. Ranging/Sidelink Positioning Protocol (RSPP) as in Solutions #26). Such Reference UE would need to be provisioned beforehand with the necessary policies and information by the LMF (of PCF) once it was still in coverage. This may include e.g. information about other Reference UEs in the area and their position, or ranging "constellation" information. It may also include information about position methods and procedures supported by the PLMN, accuracy settings, etc. The solutions for KI#1 can be extended to support the provisioning of such additional policies and information.

NOTE 2:	The details of the authorization aspects and procedures for such out-of-coverage deployment need to be studied by SA WG3.

A Reference UE that implements part of the LMF's functionality may also by itself gather the necessary information about nearby Reference UEs or ranging "constellations" by performing ranging discovery, exchange capabilities and location information or perform ranging with such nearby Reference UEs. This information can then be used to invite other Reference UEs to join a ranging session for sidelink positioning of a Target UE. It may also configure the Target UE and other Reference UEs to transmit the ranging measurements to this Reference UE, so that it can calculate the position of the Target UE based on it. For the Target UE, the ranging discovery and session setup are similar to the ranging discovery and session setup for the partial coverage case. Note that this deployment can in principle also be used for partial coverage situations.

Editor's note:	It needs to be clarified if a Reference UE with a subset of LMF functionality in this solution is the same as a SL Positioning Server UE as described in Solution #26.

In the figure below, the Target UE, Reference UE 1 and Reference UE 2 are all out of coverage. Reference UE 1 implements a subset of the LMF's functionality.

Figure 6.32.1-4: High-level overview of out-of-coverage scenario

Note that the role of being a Reference UE and in particular Located UE may not always stay the same. This depends on several factors, such as whether or not the UE has a position fix and whether or not its position is stable. For example, even if a UE includes a GNSS module in order to get a position fix, it may be out of coverage of the GNSS satellite network (e.g. driving into a tunnel or if it is deployed indoors), and hence may not be able to be a Located UE or Reference UE. Similarly if the UE is out-of-coverage of the NG-RAN it may not be able to be a Located UE or Reference UE because it cannot connect to the LMF. Also, the position of a UE may be too unstable to be used as Located UE or Reference UE (e.g. UE is moving too fast or shaking too much). If the role changes, the LMF and other UEs (e.g. Target UE, Reference UEs) involved in ranging need to be notified of such change.

NOTE 3:	How the possible roles (e.g. Reference UE, Located UE) are determined and the criteria for this are out-of-scope for SA WG2 and would need to be studied by RAN WGs.

### 6.32.2	Procedures

Editor's note:	The current procedures assume that the target UE and all the involved reference UEs are served by the same LMF and AMF in the same PLMN. It is FFS how to coordinate between LMFs in case the UEs are served by different LMFs and AMFs in different PLMNs.

Case 1: Target UE and all Reference UEs are in coverage

Figure 6.32.2-1: High-level procedure for Involving multiple Reference UEs to a ranging session for in-coverage situation

1.	The LMF may receive information about ranging "constellations" in an area from the GMLC, which may have received this information from an AF (via NEF) or LCS client (see description of Figure 6.32.2-2 for more details)

2.	The LMF collects data about potential Reference UEs/Located UEs in an area (possibly based on the ranging "constellation" information from the AF), including receiving data about the capabilities and location information for these potential Reference UEs/Located UEs, and maintains this information in a storage unit/database. This may also include keeping track of potential role changes, e.g. if a UE is not able to be a Reference UE/Located UE anymore (e.g. because it lost its position fix).

3.	The Target UE or the LMF transmit a Location Request to trigger the initiation of the ranging/sidelink positioning procedures.

4.	The Target UE initiates ranging discovery and connection setup with a discovered Reference UE (in this case Reference UE#1).

5.	The Reference UE with which a ranging connection was set up reports this to the LMF. This report may include identifiers of the Target UE and Reference UE and a session identifier).

6.	After authentication and authorization (not shown in the figure, since that is in the realm of SA WG3), the LMF determines one or more other Reference UEs to get involved in the ranging session, e.g. based on the information about ranging constellations or other Reference UEs in the area (that may have a stable position).

7.	The LMF instructs the Target UE and the Reference UEs, in this case Reference UE#1 and Reference UE#2 to invite Reference UE#2 to join the ranging session. The invitation will include a session identifier for the joint ranging session. Such message could be an extension to the Ranging/Sidelink Positioning Protocol (RSPP). The LMF may also configure the respective UEs for ranging (if this has not been done beforehand) in this step.

8.	After receiving information to invite Reference UE#2 to join the ranging session, the Target UE and Reference UE#2 can perform ranging discovery (if not done yet) and setup a ranging connection with each other, using the session identifier for the joint ranging session as input.

9.	Optionally, the other Reference UEs (in this case Reference UE#1) will also perform ranging discovery (if not done yet) and setup a ranging connection with Reference UE#2.

10.	In this step, the Target UE will perform ranging with all Reference UEs involved in the ranging session (in this case both with Reference UE#1 and with Reference UE#2) to determine a distance/angle between the respective UE. Optionally the Reference UEs are configured by the LMF to perform ranging amongst each other for additional accuracy (in this case Reference UE#1 may perform ranging with Reference UE#2).

11.	All UEs involved in the joint ranging session, in this case the Target UE and Reference UE#1 and Reference UE#2 report their ranging measurements/results to the LMF.

12. The LMF calculates the location of Target UE based on the received ranging measurements/results from all UEs involved in the joint ranging session. The LMF may report the estimated location to the Target UE or AF.

Additional details on how the LMF receives or determines information about ranging "constellations":

Figure 6.32.2-2: Example procedure of how the LMF receives ranging "constellation" information from external entity

As indicated, the LMF may receive information about ranging "constellations" in an area from an external application or an LCS client. Such information would consist at least of a list of identifiers (e.g. GPSI) of known/preferred Reference UEs/Located UEs, but may also contain additional information such as (last known) location information of such UEs, capabilities of these UEs, or area related information.

Step 1a: In the case of an LCS client, information about ranging "constellations" may be sent as part of a location request for a target UE.

Step 1b: In case of an AF, information about ranging "constellations" may be transmitted first to an NEF which forwards the information to the GMLC.

Step 2: After the GMLC has received the ranging "constellation" information it selects an AMF. This can be done in a similar way as in step 2 of solution#21, whereby information provided about the ranging "constellations" (e.g. identifier of Reference UE/Located UE, location of one or more Reference UEs/Located UEs, or area information) may be used in addition to information about a target UE. In case the GPSI is used as identifiers of Reference UEs/Located UEs, the GMLC may request the UDM to provide the SUPI of those UEs. The GMLC may also send the ranging "constellation" information to all known AMFs of the 5GC.

Step 3: The GMLC sends the information about ranging "constellations" to the selected AMF(s).

Step 4: Based on the ranging "constellation" information, the AMF selects a ranging/SL position capable LMF that serves most (if not all) of the UEs of the ranging "constellation", e.g. based on whether the area or location information provided in the ranging "constellation" information is within an LMF Service Area. The AMF may also select one or more other LMFs for which the ranging "constellation" information could be relevant, such as LMFs with partially overlapping service area or that are currently serving one or more Reference UEs/Located UEs in the constellation.

Step 5: The AMF provides the ranging "constellation" information to the selected LMF(s).

NOTE 1:	The above procedure of providing ranging "constellation" information is optional. The LMF can have its own implementation specific means to determine ranging "constellations".

Case 2: Target UE is out of coverage and at least one Reference UE is in coverage (i.e. partial coverage case)

Figure 6.32.2-3: High-level procedure for Involving multiple Reference UEs to a ranging session for partial coverage situation

Steps 1 and 2 are the same as for Case 1 (i.e. in-coverage).

3a/b.	The Target UE or the LMF may transmit a Location Request to trigger the initiation of the ranging/sidelink positioning procedures, but since the Target UE is out-of-coverage this message may be forwarded by a Reference UE that is in coverage (in this case Reference #1).

Editor's Note: It is FFS how a Location Request from the LMF reaches a Target UE that is out-of-coverage.

Editor's Note: Need to determine if this step is really necessary in case the Target UE is out-of-coverage or whether there are other means to trigger ranging/sidelink positioning, such as through policies.

Steps 4 through 6 are the same as for Case 1 (i.e. in-coverage).

7a/b.	The LMF instructs the Target UE and the Reference UEs (in this case Reference UE#1 and Reference UE#2) to invite Reference UE#2 to join the ranging session. The invitation will include a session identifier for the joint ranging session. The LMF may also configure the respective UEs for ranging (if this has not been done beforehand) in this step. Since the Target UE is out-of-coverage the Reference UE that is in coverage (in this case Reference #1) may provide or forward this message to the Target UE on behalf of the LMF. Similar if Reference UE#2 is out-of-coverage.

NOTE 2:	For the forwarding of the messages, e.g. (a subset of) ProSe UE-to-Network Relay functionality or some gateway functionality (e.g. for repackaging RSPP from PC5 to Uu) could be considered. This can be decided during normative phase and/or as part of the conclusions depending on other partial coverage solutions.

Steps 8 through 10 are the same as for Case 1 (i.e. in-coverage).

11a/b/c.	All UEs involved in the joint ranging session, in this case the Target UE and Reference UE#1 and Reference UE#2 need to report their ranging measurements/results to the LMF. However, since the Target UE is out-of-coverage, it may report its ranging measurements/results to the Reference UE that is in coverage (in this case Reference UE#1), which may (collect and) forward the ranging measurements/results to the LMF. Similar if Reference UE#2 is out-of-coverage.

Step 12 is the same as for Case 1 (i.e. in-coverage).

Case 3: Target UE as well as the Reference UEs are out of coverage

Figure 6.32.2-4: High-level procedure for Involving multiple Reference UEs to a ranging session for out-of-coverage situation

Step 0 is the same as step 1 for Case 1 (i.e. in-coverage) and Case 2 (i.e. partial coverage).

1.	Optionally the LMF configures the Reference UE that supports a subset of the LMF functionality (i.e. Reference UE#1 in this case), when that Reference UE is in coverage.

Step 2 is the same as for Case 1 (i.e. in coverage) and Case 2 (i.e. partial coverage), but is now executed by the Reference UE that supports a subset of the LMF functionality (i.e. Reference UE#1 in this case).

3.	The Target UE or the Reference UE that supports a subset of the LMF functionality (i.e. Reference UE#1 in this case) may transmit a Location Request to trigger the initiation of the ranging/sidelink positioning procedures.

Editor's note:	Need to determine if this step is really necessary in case the Target UE is out-of-coverage or whether there are other means to trigger ranging/sidelink positioning, such as through policies.

Step 4 is the same as for Case 1 (i.e. in-coverage) and Case 2 (i.e. partial coverage).

Step 5 from Case 1 and Case 2 (i.e. reporting ranging session to the LMF) is omitted since the Reference UE#1 itself supports the respective LMF functionality.

Step 6 is the same as for Case 1 (i.e. in-coverage) and Case 2 (i.e. partial coverage), but is now executed by the Reference UE that supports a subset of the LMF functionality (i.e. Reference UE#1 in this case).

7.	The Reference UE that supports a subset of the LMF functionality (i.e. Reference UE#1 in this case) instructs the Target UE and the Reference UEs (in this case Reference UE#1 and Reference UE#2) to invite Reference UE#2 to join the ranging session. The invitation will include a session identifier for the joint ranging session. The Reference UE that supports a subset of the LMF functionality (i.e. Reference UE#1 in this case) may also configure the respective UEs for ranging (if this has not been done beforehand) in this step.

Steps 8 through 10 are the same as for Case 1 (i.e. in-coverage) and Case 2 (i.e. partial coverage).

11.	All UEs involved in the joint ranging session, in this case the Target UE and Reference UE#1 and Reference UE#2 need to report their ranging measurements/results to the Reference UE that supports a subset of the LMF functionality (i.e. Reference UE#1 in this case).

Step 12 is the same as for Case 1 (i.e. in-coverage) and Case 2 (i.e. partial coverage), but is now executed by the Reference UE that supports a subset of the LMF functionality (i.e. Reference UE#1 in this case).

### 6.32.3	Impacts on services, entities and interfaces

Reference UE:

-	Support for ranging procedures (e.g. discovery, performing ranging signalling/measurements, etc.).

-	Support of RSPP.

-	Optional support of a subset of LMF functionality, which may include:

-	collect ranging measurements/results and calculate position

-	configure Target UE and other Reference UEs for ranging

-	inviting other Reference UE to a ranging session

-	collecting/maintaining/providing information about other Reference UEs/ranging "constellation" information.

-	Optional support for forwarding messages from a Target UE and other Reference UEs to the LMF and vice-versa.

Target UE:

-	Support for ranging procedures (e.g. discovery, performing ranging signalling/measurements, etc.).

-	Support of RSPP.

LMF:

-	Support of RSPP.

-	Ability to collect ranging measurements/results and calculate position.

-	Ability to invite other Reference UEs to a ranging session.

-	Collecting/maintaining/providing information about other Reference UEs/ranging "constellation" information.

-	Optional support to provision Reference UEs that support a subset of LMF functionality.

GMLC:

-	Optional support for receiving ranging "constellation" information via NEF or LCS client.

-	Optional support for interpreting "constellation" information to select one or more AMFs.

AMF:

-	Optional support for receiving ranging "constellation" information from GMLC.

-	Optional support for interpreting "constellation" information to select one or more LMFs.

## 6.33	Solution #33: Common Ranging/Sidelink positioning service exposure

### 6.33.1	General

This solution is to address KI#6 and KI#7.

The overall procedure of Ranging/Sidelink positioning consists of the following steps:

-	Ranging/Sidelink positioning request/response (e.g. received from/sent to SL Positioning Client UE, AMF, LMF).

-	Measurement between 2 UEs or between one UE and multiple UEs.

-	Result calculation (e.g. at SL Positioning Server UE or at LMF, or at both).

A SL Positioning Server UE supports at least one of the following functionalities:

-	Receive/send Ranging/Sidelink positioning request/response from/to UE or LMF.

-	Result calculation.

Either service is triggered by UE or Application server, result calculation can be performed at SL Positioning Server UE, at LMF, or at both. However, involving LMF for the result calculation is expected to be avoided as much as possible to reduce the delay and network complexity. Before it is determined to use LMF, try to perform the procedure on the UE side first. If at some step UE determines to sort help from network, then proper LMF is selected and involved.

### 6.33.2	Functional descriptions

In a Ranging/Sidelink positioning procedure, SL Positioning Server UE functionalities (i.e. Receive/send Ranging/Sidelink positioning request/response from/to UE or LMF and Result calculation) have to be performed. If neither Ranging/Sidelink positioning UEs (i.e. reference UE and target UE) supports SL Positioning Server UE functionalities, a SL Positioning Server UE has to be discovered. The Ranging/Sidelink positioning UEs interact with the SL Positioning Server UE for transmitting the measurement data and the result.

However, if no SL Positioning Server UE in proximity can be discovered, the Ranging/Sidelink positioning UE has to notify the network to select a proper LMF for result calculation.

Editor's note:	For the Ranging/Sidelink Positioning service request received from GMLC, whether to select a LMF or a SL Positioning Server UE to perform result calculation is FFS.

In a network based solution, Ranging/Sidelink positioning UE receives the Ranging/Sidelink positioning request from its serving AMF. The AMF may receive a Ranging/Sidelink positioning request from LMF, a SL Positioning Client UE, GMLC, or NEF.

In a non-network based solution, Ranging/Sidelink positioning UE receives the Ranging/Sidelink positioning request from a SL Positioning Client UE directly over PC5.

In both cases, when Ranging/Sidelink positioning UE receives the request, it starts to discover the other Ranging/Sidelink positioning UE. After the other UE is successfully discovered, they exchange capabilities.

If SL Positioning Server UE functionalities are supported by either one, the further measurements and result calculations will only be performed by the 2 UEs; otherwise, the Ranging/Sidelink positioning UE discovers a SL Positioning Server UE, using either Mode A or Mode B ProSe discovery by indicating the capability of SL Positioning Server UE for the discovery, so that only a UE supporting SL Positioning Server functionalities is discovered.

Ranging/Sidelink positioning UE may be provisioned or pre-configured with a SL Positioning Server UE ID, in that case, the Ranging/Sidelink positioning UE discovers the SL Positioning Server UE based on the preconfigured ID. If multiple SL Positioning Server UEs are discovered, the SL Positioning Server UE is selected based on their capabilities, the pre-configured SL Positioning Server UE ID, the distance, and the signaling strength.

### 6.33.3	Procedures

Figure 6.33.3-1: Procedure of common Ranging/sidelink positioning exposure (network based solution)

1.	UE1's serving AMF receives a Ranging/Sidelink positioning service request from the SL Positioning Client UE or a 5GC NF

2.	If no operator policy indicating the use of LMF, the AMF sends the request to UE1.

3.	UE1 discovers UE2. UE1 and UE2 exchange their capabilities to determine if it is necessary to use a SL Positioning Server UE:

-	if neither UE1 nor UE2 supports SL Positioning Server functionalities, UE1 or UE2 discovers a SL Positioning Server UE.

-	otherwise, no need to discover the SL Positioning server UE, and step 4 is skipped.

Editor's note:	The security issues due to the involvement of SL Positioning server UE in the Ranging/Sidelink Positioning procedure will be evaluated by SA WG3.

4.	UE1 or UE2 or both discover a SL Positioning Server UE using either Mode A or Mode B ProSe discovery by indicating the capability of SL Positioning Server UE for the discovery. If multiple SL Positioning Server UEs are discovered, the SL Positioning Server UE is selected based on their capabilities, the pre-configured SL Positioning Server UE ID, the distance, and the signaling strength.

Option A (start)

5.	If a SL Positioning Server UE is successfully selected, Ranging/Sidelink positioning measurement and result calculation are performed among UE1, UE2 and the selected SL Positioning Server UE (if needed).

6.	UE1 responds to AMF with the Ranging/Sidelink positioning result.

Option A (end)

Option B (start)

7.	If UE1 and UE2 fail in discovering and selecting a SL Positioning Server UE, UE1 notifies AMF that no SL Positioning Server UE is available.

8.	AMF determines to involve LMF.

9.	A LMF is selected by the AMF. The selected LMF triggers UE1 or UE2 to perform Ranging/Sidelink positioning measurement. The measurement data is sent to LMF for result calculation.

Option B (end)

10.	AMF sends the Ranging/Sidelink positioning result to the SL Positioning Client UE or a 5GC NF.

Figure 6.33.3-2: Procedure of common Ranging/sidelink positioning exposure (non-network based solution)

1.	UE1 receives a Ranging/Sidelink positioning service request from the SL Positioning Client UE

2.	UE1 discovers UE2. UE1 and UE2 exchange their capabilities to determine if it is necessary to use a SL Positioning Server UE:

-	if neither UE1 nor UE2 supports SL Positioning Server functionalities, UE1 or UE2 discovers a SL Positioning Server UE.

-	otherwise, no need to discover the SL Positioning Server UE, and step 3 is skipped.

3.	UE1 or UE2 or both discover a SL Positioning Server UE using either Mode A or Mode B ProSe discovery by indicating the capability of SL Positioning Server UE for the discovery. If multiple SL Positioning Server UEs are discovered, the SL Positioning Server UE is selected based on their capabilities, the pre-configured SL Positioning Server UE ID, the distance, and the signaling strength.

Option A (start)

4.	If a SL Positioning Server UE is successfully selected, Ranging/Sidelink positioning measurement and result calculation are performed among UE1, UE2 and the selected SL Positioning Server UE (if needed).

5.	UE1 responds to AMF with the Ranging/Sidelink positioning result

Option A (end)

Option B (start)

6.	If UE1 and UE2 fail in discovering and selecting a SL Positioning Server UE, UE1 send the Ranging/Sidelink positioning request to the AMF, which may include an indication that no SL Positioning Server UE is available.

7.	AMF determines to involve LMF.

8.	A LMF is selected by the AMF. The selected LMF triggers UE1 or UE2 to perform Ranging/Sidelink positioning measurement. The measurement data is sent to LMF for result calculation.

9.	AMF sends the Ranging/Sidelink positioning result to UE1.

10.	AMF sends the Ranging/Sidelink positioning result to the SL Positioning Client UE.

Option B (end)

### 6.33.4	Impacts on services, entities, and interfaces

The solution has impacts in the following entities and interfaces:

-	UE: supports initiating Ranging/SL Positioning procedure; receives Ranging/SL Positioning service request from AMF or the third UE; sends Ranging/SL Positioning result to AMF or the 3rd UE, SL Positioning server functionalities; SL Positioning Server UE discovery and selection; notification to 5GC that no SL Positioning Sever UE is available.

-	AMF: receives and forwards the Ranging/SL Positioning service request to the UE1; receives and forwards the Ranging/SL Positioning result to the SL Positioning Client UE, handle notification about no available SL Positioning Sever UE.

-	NAS: supports transmission of Ranging/SL Positioning service request and Ranging/SL Positioning result; notification to 5GC that no SL Positioning Sever UE is available.

-	PC5: supports transmission of Ranging/SL Positioning service request and Ranging/SL Positioning result.

## 6.X	Solution #x: title of the solution

Editor's note:	This clause describes a solution addressing one or more key issues identified in clause 5. The structure of the clauses can be adjusted. The list of key issues which this solution attempts to resolve should be clearly indicated.

### 6.X.1	General

### 6.X.2	Functional descriptions

### 6.X.3	Procedures

### 6.X.4	Impacts on services, entities, and interfaces

# 7	Evaluation

Editor's note:	This clause provides the evaluations of the solutions of clause 6.

# 8	Conclusions

Editor's note:	This clause provides the conclusions for the study.

Annex A:
<Informative annex title >

Annex B:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-02 | SA2#149e | S2-2201607 | - | - | - | FS_Ranging_SL TR skeleton | 0.0.0 |
| 2022-02 | SA2#149e | S2-2201109 | - | - | - | Proposed TR Scope | 0.1.0 |
| 2022-02 | SA2#149e | S2-2201608 | - | - | - | Proposal on Definition and Abbreviations | 0.1.0 |
| 2022-02 | SA2#149e | S2-2201609 | - | - | - | Proposed Architecture Assumptions and Requirements | 0.1.0 |
| 2022-02 | SA2#149e | S2-2201610 | - | - | - | Key Issue: Support of authorisation, policy and parameter provisioning for ranging services | 0.1.0 |
| 2022-02 | SA2#149e | S2-2201611 | - | - | - | Key Issue: Ranging service procedure with the assistance of another UE. | 0.1.0 |
| 2022-02 | SA2#149e | S2-2201612 | - | - | - | New KI on Ranging device discovery | 0.1.0 |
| 2022-02 | SA2#149e | S2-2201613 | - | - | - | New key issue on RAT independent control signalling | 0.1.0 |
| 2022-02 | SA2#149e | S2-2201614 | - | - | - | KI for Sidelink positioning for Partial Network Coverage | 0.1.0 |
| 2022-02 | SA2#149e | S2-2201615 | - | - | - | KI#X: Ranging and sidelink positioning service exposure to a UE | 0.1.0 |
| 2022-02 | SA2#149e | S2-2201616 | - | - | - | Ranging and sidelink positioning service exposure to application server | 0.1.0 |
| 2022-04 | SA2#150e | S2-2203337 | - | - | - | Proposed new KI on Service Authorization to NG-RAN | 0.2.0 |
| 2022-04 | SA2#150e | S2-2203338 | - | - | - | Solution for KI#1 on authorization and policy and parameter provisioning for ranging services | 0.2.0 |
| 2022-04 | SA2#150e | S2-2203339 | - | - | - | Proposed new solution on authorization and policy/parameter provisioning to UE | 0.2.0 |
| 2022-04 | SA2#150e | S2-2203340 | - | - | - | Solution for KI#4 on direct ranging operation | 0.2.0 |
| 2022-04 | SA2#150e | S2-2203341 | - | - | - | New solution for Ranging devices discovery and ranging procedure | 0.2.0 |
| 2022-04 | SA2#150e | S2-2203342 | - | - | - | KI#3, KI#4, New Sol: Solution to support ranging/sidelink positioning | 0.2.0 |
| 2022-04 | SA2#150e | S2-2203343 | - | - | - | Network assisted Ranging and Sidelink Positioning | 0.2.0 |
| 2022-04 | SA2#150e | S2-2203344 | - | - | - | Solution for Network assisted SL positioning | 0.2.0 |
| 2022-04 | SA2#150e | S2-2203345 | - | - | - | Network assisted Sidelink Positioning for In Network Coverage and Partial Network Coverage | 0.2.0 |
| 2022-04 | SA2#150e | S2-2203346 | - | - | - | Solution for KI#6 on ranging and sidelink positioning service exposure to UE | 0.2.0 |
| 2022-04 | SA2#150e | S2-2203347 | - | - | - | New solution for Ranging service exposure to UE | 0.2.0 |
| 2022-04 | SA2#150e | S2-2203348 | - | - | - | Solution for UE using exposure API | 0.2.0 |
| 2022-04 | SA2#150e | S2-2203349 | - | - | - | New Solution on Ranging Service Exposure towards AF | 0.2.0 |
| 2022-04 | SA2#150e | S2-2203350 | - | - | - | Proposed new solution on service initiated by Application server or 5GC NF | 0.2.0 |
| 2022-04 | SA2#150e | S2-2203351 | - | - | - | New Solution on reference architecture for Sidelink Positioning and Ranging services | 0.2.0 |
| 2022-06 | SA2#151e | S2-2205047 | - | - | - | Proposed new term on Network assisted UE. | 0.3.0 |
| 2022-06 | SA2#151e | S2-2205048 | - | - | - | Proposed general reference architecture. | 0.3.0 |
| 2022-06 | SA2#151e | S2-2205049 | - | - | - | KI#2 clarifications on multiple UEs. | 0.3.0 |
| 2022-06 | SA2#151e | S2-2204659 | - | - | - | KI#3 Alignment on terminology and definitions. | 0.3.0 |
| 2022-06 | SA2#151e | S2-2205050 | - | - | - | KI #1, KI #8, Sol #1: Update to add the trigger condition for authorisation and policy/parameter provisioning to UE. | 0.3.0 |
| 2022-06 | SA2#151e | S2-2205051 | - | - | - | KI#8, New solution: Service Authorization to NG-RAN for Ranging/Sidelink Positioning. | 0.3.0 |
| 2022-06 | SA2#151e | S2-2205052 | - | - | - | KI#2, New Sol: Ranging service operation with assistant UE. | 0.3.0 |
| 2022-06 | SA2#151e | S2-2205053 | - | - | - | KI#2&6, Sol#10_Updates to clarify the ranging exposure procedure to UE. | 0.3.0 |
| 2022-06 | SA2#151e | S2-2205054 | - | - | - | KI#2, New solution: Ranging/Sidelink positioning with the assistance of assistant UE. | 0.3.0 |
| 2022-06 | SA2#151e | S2-2205055 | - | - | - | KI#3&4, Sol#4_Updates to remove ENs. | 0.3.0 |
| 2022-06 | SA2#151e | S2-2205056 | - | - | - | Network triggered ranging device discovery. | 0.3.0 |
| 2022-06 | SA2#151e | S2-2204478 | - | - | - | KI #3, Sol #5: Update to remove ENs. | 0.3.0 |
| 2022-06 | SA2#151e | S2-2205057 | - | - | - | KI# 4, New Solution: Support of Sidelink Positioning and Ranging signalling over different PC5 RATs. | 0.3.0 |
| 2022-06 | SA2#151e | S2-2205058 | - | - | - | KI#5, Sol#7: Update to address EN. | 0.3.0 |
| 2022-06 | SA2#151e | S2-2205059 | - | - | - | KI#5, New Sol: Network assisted Sidelink positioning with scheduled location time. | 0.3.0 |
| 2022-06 | SA2#151e | S2-2205060 | - | - | - | Sol#6_Updates to remove ENs. | 0.3.0 |
| 2022-06 | SA2#151e | S2-2205061 | - | - | - | KI #5, Sol #8: Update to remove ENs. | 0.3.0 |
| 2022-06 | SA2#151e | S2-2205062 | - | - | - | KI#5, New solution: Network assisted sidelink positioning for partial coverage. | 0.3.0 |
| 2022-06 | SA2#151e | S2-2205063 | - | - | - | KI #6, Sol #9: Update to add selection of initiator UE from UE1 and UE2 by LMF. | 0.3.0 |
| 2022-06 | SA2#151e | S2-2205064 | - | - | - | KI #6, KI #7, New Sol: Combine Uu positioning capability and/or Ranging/SL positioning capability for the Ranging/SL positioning service. | 0.3.0 |
| 2022-06 | SA2#151e | S2-2205065 | - | - | - | New solution for Ranging service exposure to server. | 0.3.0 |
| 2022-06 | SA2#151e | S2-2205066 | - | - | - | KI#7: Sol#12 updates to address EN. | 0.3.0 |
| 2022-06 | SA2#151e | S2-2204671 | - | - | - | KI#7, New solution: service initiated by Application server over user plane. | 0.3.0 |
| 2022-06 | SA2#151e | S2-2204679 | - | - | - | KI#6, New solution: service initiated by third party UE. | 0.3.0 |
| 2022-06 | SA2#151e | S2-2205067 | - | - | - | New Solution: Functional Split and Positioning Protocol for SL Positioning. | 0.3.0 |
| 2022-06 | SA2#151e | S2-2204672 | - | - | - | Proposed editorial changes. | 0.3.0 |
| 2022-06 | SA2#151e | S2-2205068 | - | - | - | KI#2/3/4/5/6/7, New solution: LMF selection for Ranging/Sidelink positioning. | 0.3.0 |
| 2022-06 | SA2#151e | S2-2205069 | - | - | - | KI#2/3/4/5/6/7, New solution: using LMF for Ranging/Sidelink positioning. | 0.3.0 |
| 2022-09 | SA2#152e | S2-2207117 | - | - | - | General: Clarification on Location Server UE and peer-to-peer operation | 0.4.0 |
| 2022-09 | SA2#152e | S2-2207118 | - | - | - | Updates to terminologies | 0.4.0 |
| 2022-09 | SA2#152e | S2-2207119 | - | - | - | KI#2, New Solution: Service continuity of periodic ranging with assistant UE | 0.4.0 |
| 2022-09 | SA2#152e | S2-2206375 | - | - | - | KI#2, Sol#16_Update to support multiple assistant UEs case | 0.4.0 |
| 2022-09 | SA2#152e | S2-2207120 | - | - | - | KI#2 new solution Ranging service operation with the assistant UE | 0.4.0 |
| 2022-09 | SA2#152e | S2-2207121 | - | - | - | KI#5, solution#8: update solution to resolve some Editor s notes | 0.4.0 |
| 2022-09 | SA2#152e | S2-2207122 | - | - | - | KI#5, New solution: enabling SL positioning for the UE out of coverage | 0.4.0 |
| 2022-09 | SA2#152e | S2-2206918 | - | - | - | KI#5 Sol#6 update assumption on NAS connection between Target UE and AMF | 0.4.0 |
| 2022-09 | SA2#152e | S2-2207123 | - | - | - | KI#4, KI#5 New Solution Involving Multiple Reference UEs for Sidelink positioning | 0.4.0 |
| 2022-09 | SA2#152e | S2-2207124 | - | - | - | KI #6, Sol #9: Update with selection of initiator UE by LMF | 0.4.0 |
| 2022-09 | SA2#152e | S2-2207125 | - | - | - | KI#7, KI#5: Sol#12 updates addressing ENs | 0.4.0 |
| 2022-09 | SA2#152e | S2-2207126 | - | - | - | KI#6, KI#7 New Solution: Common Ranging/Sidelink positioning service exposure | 0.4.0 |
| 2022-09 | SA2#152e | S2-2207127 | - | - | - | KI#6, Solution#25: Update to include how SUPI of Ranging/Sidelink positioning UE is determined | 0.4.0 |
| 2022-09 | SA2#152e | S2-2207128 | - | - | - | KI#3&KI#5, Sol#14: Updates to add 5G DDNMF services | 0.4.0 |