| 3GPP TR 23.947 V0.1.0 (2025-08) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Guidelines for 3GPP Application Enablement usage; (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	4

1	Scope	5

2	References	5

3	Definitions of terms, symbols and abbreviations	5

3.1	Terms	5

3.2	Symbols	6

3.3	Abbreviations	6

4	Overview of Frameworks	7

4.1	Introduction	7

4.2	Common API Framework (CAPIF)	7

4.3	Edge Application Enablement (EDGEAPP)	7

4.4	Service Enabler Architecture Layer (SEAL)	7

4.4	Inter-framework Relationships Overview	7

5	Use Cases	7

5.1	Use case X	7

5.1.1	Use case Description	7

5.1.2	Use case Realisation over SA6 Frameworks	7

5.1.3	Use case Flows	7

6	Developer Guidelines	8

6.1	Introduction	8

6.2	Cross-Framework Integration	8

6.3	Example Application Flows	8

6.4	Multi-CAPIF Considerations	8

7	Security Considerations	8

Annex A: Infrastructure Topology Diagrams Examples	9

CAPIF+EDGEAPP Topology	9

CAPIF+SEAL Topology	9

CAPIF+EDGEAPP+SEAL Topology	9

Annex B: Change history	10

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

Emerging services from verticals such as connected vehicles (V2X), unmanned aerial vehicles (UAVs) and smart factories require low-latency, high-bandwidth, and context-aware services delivered close to the end-user. SA6 in 3GPP has developed several enablement frameworks to support these requirements:

-	CAPIF (3GPP TS 23.222 [2]) defines a common and secure way to expose and consume network and service APIs across multiple domains and stakeholders.

-	EDGEAPP (3GPP TS 23.558 [4]) specifies the architecture for enabling and orchestrating applications at the network edge, including discovery, service continuity and mobility support.

-	SEAL (3GPP TS 23.434 [6]) provides reusable service enablers that can be consumed by vertical applications via standardised APIs. Examples of these common services are Group management or Location management.

This TR provides use case examples oriented to Application developers for understanding how SA6 enablement frameworks work together and what APIs are essential for developing their applications when dealing with Operators/Service Providers infrastructure that follows SA6 specifications.

# 1	Scope

This Technical Report (TR) explains how application (App) developers can leverage the services from enablement frameworks, namely Common API Framework (CAPIF), Edge Application Enablement (EDGEAPP), and the Service Enabler Architecture Layer (SEAL) to create their software.

To facilitate understanding, this study presents a set of representative deployment options. These are provided as examples to illustrate common operator scenarios, but they do not limit the options that the frameworks offer. The TR will cover the linkage between the available services with different vertical use cases, to guide App developers on how to use the different services to fulfil their use cases.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs (CAPIF); (Stage 2)".

[3]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs (CAPIF); (Stage 3)".

[4]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[5]	3GPP TS 29.558: "Enabling Edge Applications; Application Programming Interface (API) specification; (Stage 3)".

[6]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[7]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs".

[8]	3GPP TS 33.558: "Security aspects of enhancement of support for enabling edge applications".

[9]	3GPP TS 33.434: "Security aspects of Service Enabler Architecture Layer (SEAL) for verticals".

[10]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[11]	IETF RFC 7519: "JSON Web Token (JWT)".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Application Client (AC): An application component, typically hosted on User Equipment (UE), that consumes services provided by an EAS (as defined in 3GPP TS 23.558 [4]).

Application Context: A set of data about the Application Client that resides in the Edge Application Server (as defined in 3GPP TS 23.558 [4]).

CAPIF API Invoker: An entity that consumes APIs exposed via CAPIF (as defined in 3GPP TS 23.222 [2]).

CAPIF API Provider: An entity that offers APIs via CAPIF (as defined in 3GPP TS 23.222 [2]).

Common API Framework (CAPIF): A 3GPP framework that provides common functions for API exposure, onboarding, discovery, and authorization across multiple domains and stakeholders (as defined in 3GPP TS 23.222 [2]).

Edge Application Server (EAS): An application component hosted on an edge node that provides services to ACs (as defined in 3GPP TS 23.558 [4]).

Edge Enabler Client (EEC): A client-side function, co-located with the AC, that interacts with the Edge Enabler Server (EES) for discovery, registration, and mobility support (as defined in 3GPP TS 23.558 [4]).

Edge Enabler Server (EES): A network function that provides EDGEAPP control plane services to EECs, including EAS discovery, mobility handling, and service continuity (as defined in 3GPP TS 23.558 [4]).

Edge Computing Service Provider (ECSP): An operator or provider responsible for deploying and operating EDGEAPP functions (ECS, EES, EAS) in the network (as defined in 3GPP TS 23.558 [4]).

Service Enabler Architecture Layer (SEAL): A framework providing reusable service enablers such as location, group communications, and messaging, available to vertical applications (as defined in 3GPP TS 23.434 [6]).

SEAL Server: A server implementing SEAL service enabler functions, possibly hosted centrally or at the edge, and accessible via CAPIF (as defined in 3GPP TS 23.434 [6]).

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AC	Application Client

AEF	API Exposing Function

AMF	API Managing Function

APF	API Publishing Function

CAPIF		Common API Framework

CCF	CAPIF Core Function

EAS	Edge Application Server

ECSP	Edge Computing Service Provider

EEC	Edge Enabler Client

EES	Edge Enabler Server

ECS	Edge Configuration Server

EDGEAPP	Edge Application Enablement

JSON	JavaScript Object Notation

OAuth	Open Authorization

REST	Representational State Transfer

SEAL	Service Enabler Architecture Layer

# 4	Overview of Frameworks

## 4.1	Introduction

## 4.2	Common API Framework (CAPIF)

This clause will provide the CAPIF role in secure API exposure, onboarding, and discovery.

## 4.3	Edge Application Enablement (EDGEAPP)

This clause will provide the EDGEAPP role in application lifecycle and orchestration at the edge.

## 4.4	Service Enabler Architecture Layer (SEAL)

This clause will provide the SEAL role in the provision of reusable service enablers for vertical applications.

## 4.4	Inter-framework Relationships Overview

This clause will describe the role of each framework can provide in an overall framework.

# 5	Use Cases

Each use case is expected to include end-to-end service flow and infrastructure details (e.g., number and placement of edge nodes, optional use of SEAL services for a single PLMN domain)

## 5.1	Use case X

### 5.1.1	Use case Description

### 5.1.2	Use case Realisation over SA6 Frameworks

### 5.1.3	Use case Flows

# 6	Developer Guidelines

## 6.1	Introduction

## 6.2	Cross-Framework Integration

CAPIF roles definition for EDGEAPP and SEAL entities

## 6.3	Example Application Flows

The following flows will be considered “CAPIF + EDGEAPP”, “CAPIF + SEAL” and “CAPIF + EDGEAPP + SEAL”

## 6.4	Multi-CAPIF Considerations

Relevant information to consider when several CAPIF are deployed

# 7	Security Considerations

Aspects such as identity management across frameworks, secure onboarding and trust models, framework-specific and integrated authorization will be considered

Annex A:
Infrastructure Topology Diagrams Examples

Will show CAPIF+ EDGEAPP, CAPIF+SEAL and CAPIF+EDGEAPP+SEAL

## CAPIF+EDGEAPP Topology

## CAPIF+SEAL Topology

## CAPIF+EDGEAPP+SEAL Topology

Annex B:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-08 | SA6#68 | S6-253778 |  |  |  | TR Initial Version | 0.0.0 |
| 2025-08 | SA6#68 | S6-253670 S6-253778 S6-253727 |  |  |  | References Scope Introduction | 0.1.0 |