| 3GPP TS 24.559 V0.3.0 (2023-11) |  |
| --- | --- |
| Technical Specification¦Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals Application Data Analytics Enablement Service; Stage 3; (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Abbreviations	8

4	General description	8

4.1	Overview	8

5	Functional entities	8

5.1	Application data analytics enablement server (ADAE-S)	8

5.2	Application data analytics enablement client (ADAE-C)	8

6	Application data analytics enablement service procedures	9

6.1	General	9

6.2	Application performance analytics	9

6.2.1	General	9

6.2.2	Analytic subscription configuration procedure	9

6.3	Network slice related application data analytics	10

6.3.1	General	10

6.4	Network slice configuration recommendation based on application analytics	10

6.4.1	General	10

6.5	Edge load analytics	10

6.5.1	General	10

6.6	UE-to-UE communications performance analytics	10

6.6.1	General	10

6.7	Location accuracy analytics	10

6.7.1	General	10

6.8	Service application programming interface (API) analytics	10

6.8.1	General	10

6.9	Application performance analytics	11

6.9.1	General	11

7	API Definitions	11

7.1	ADAE_ServiceConfiguration API	11

7.1.1	Introduction	11

7.1.2	Usage of HTTP and common API related aspects	11

7.1.3	Resources	11

7.1.3.1	Overview	11

7.1.3.2	Resource: Application performance event subscription	13

7.1.3.2.1	Description	13

7.1.3.2.2	Resource definition	13

7.1.3.2.3	Resource standard methods	13

7.1.3.2.3.1	POST	13

7.1.3.2.4	Resource custom operations	14

7.1.3.3	Resource: UE-to-UE session performance analytics	14

7.1.3.3.1	Description	14

7.1.3.3.2	Resource definition	14

7.1.3.3.3	Resource standard methods	14

7.1.3.3.4	Resource custom operations	15

7.1.3.3.4.1	Overview	15

7.1.3.3.4.2	POST	15

7.1.3.4	Resource: Edge load event subscription	15

7.1.3.4.1	Description	15

7.1.3.4.2	Resource definition	15

7.1.3.4.3	Resource standard methods	16

7.1.3.4.3.1	POST	16

7.1.3.5	Resource: Service experience	16

7.1.3.5.1	Description	16

7.1.3.5.2	Resource definition	16

7.1.3.5.3	Resource standard methods	17

7.1.3.5.3.1	POST	17

7.1.3.5.4	Resource custom operations	17

7.1.3.5.4.1	Overview	17

7.1.3.5.4.2	Operation: POST	17

7.1.3.5.4.3	Operation: POST	18

7.1.4.2	Application performance event notification	18

7.1.4.2.1	Description	18

7.1.4.2.2	Notification definition	18

7.1.4.3	Edge load event notification	19

7.1.4.3.1	Description	19

7.1.4.3.2	Notification definition	19

7.1.4.4	Service experience report event notification	20

7.1.4.4.1	Description	20

7.1.4.4.2	Notification definition	20

7.1.4	Notifications	21

7.1.4.1	General	21

7.1.5	Data model	21

7.1.5.1	General	21

7.1.5.2	Structured data types	22

7.1.5.2.1	Introduction	22

7.1.5.2.2	Type: Ue2UePerfReq	22

7.1.5.2.3	Type: Ue2UePerfResp	22

7.1.5.2.4	Type: ConfigRepTrigger	23

7.1.5.2.5	Type: PushSrvExpInfo	23

7.1.5.2.6	Type: PullSrvExpInfo	23

7.1.5.2.7	Type: SrvExpInfoRep	24

7.1.6	Error Handling	24

7.1.6.1	General	24

7.1.6.2	Protocol Errors	24

7.1.6.3	Application Errors	24

7.1.7	Feature Negotiation	24

Annex <A> (normative): HTTP resource representation and encoding	25

Annex <B> (Normative): CoAP resource representation and encoding	26

Annex <C> (informative): Change history	27

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the protocol aspects of ADAE of SEAL services. The protocol aspects specify the UE supporting the client functionality of the ADAE SEAL services and the network supporting the server functionality of ADAE SEAL services, where the client functionality and server functionality are specified in 3GPP TS 23.436 [2].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.436: " Procedures for Application Data Analytics Enablement Service".

[X]	IETF RFC 4825: "The Extensible Markup Language (XML) Configuration Access Protocol (XCAP)".

[Y]	OMA OMA-TS-XDM_Core-V2_1-20120403-A: "XML Document Management (XDM) Specification".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

ADAE client: An entity that provides the client side functionalities corresponding to the ADAE.

ADAE server: An entity that provides the server side functionalities corresponding to the ADAE.

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.436 [2] apply:

ADAE service

SEAL server

SEAL service

VAL application

VAL server

VAL service

VAL client

Vertical

Vertical application

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ADAE	Application Data Analytics Enablement

ADAES	Application Data Analytics Enablement Service

ADAE-C	Application Data Analytics Enablement Client

ADAE-S	Application Data Analytics Enablement Server

A-ADRF	Application layer Analytical Data Repository Function

API	Application Programming Interface

EAS		Edge Application Server

EDN		Edge Data Network

EES		Edge Enabler Server

NWDAF	Network Data Analytics Function

SEAL		Service Enabler Architecture Layer

UE		User Equipment

VAL	Vertical Application Layer

# 4	General description

## 4.1	Overview

Application data analytics enablement service enables an application data analytics enablement client (ADAE-C) and a vertical application layer (VAL) server to communicate with an application data analytics enablement server (ADAE-S).

# 5	Functional entities

## 5.1	Application data analytics enablement server (ADAE-S)

The ADAE-S is a functional entity with a unique identity, ADAE-S ID, in the PLMN and provides data analytics to administer the operation and performance of one or more VAL applications.

To be compliant with the HTTP procedures in the present document the ADAE-S:

a)	shall support the role of XCAP server as specified in IETF RFC 4825 [X]; and

b)	shall support the role of XDMS as specified in OMA OMA-TS-XDM_Core-V2_1 [Y].

## 5.2	Application data analytics enablement client (ADAE-C)

The ADAE-C functional entity with a unique identity, ADAE-C ID, and act as the VAL application client which provides data analytics for managing network slice capabilities.

To be compliant with the HTTP procedures in the present document the ADAE-C:

a)	shall support the role of XCAP client as specified in IETF RFC 4825 [X]; and

b)	shall support the role of XDMC as specified in OMAOMA-TS-XDM_Core-V2_1 [Y].

# 6	Application data analytics enablement service procedures

## 6.1	General

The clause includes the roles of the ADAE-S and the ADAE-C when implementing the analytics provided by the application data analytics enablement service.

## 6.2	Application performance analytics

### 6.2.1	General

These clauses describe the procedures on the ADAE server and ADAE client side when a consumer such as a VAL server subscribes to the VAL performance analytics. The VAL performance analytics are performed based on:

a)	ongoing VAL sessions; or

b)	ongoing VAL sessions and data from VAL server or database.

### 6.2.2	Analytic subscription configuration procedure

Upon receipt an HTTP POST request with a Request-URI according to the pattern "{apiRoot}/adae-sc/<apiVersion>/data-event/{dataEventId}/configurtions/{configurationId}" where:

a)	"dataEventId" identifying the identity of an event for the VAL performance analytics; and

b)	"configurationId" identifying the analytics subscription configuration; and

with a body containing AnalytSubscripConfig data type incuding:

a)	analytics-filter;

b)	analytics-type;

c)	val-service-id;

d)	optionally val-ue-list with one or more VAL UE IDs;

e)	optionally val-server-id;

f)	optionally data-producer-profile;

g)	optionally confidence-level;

h)	optionally area; and

i)	optionally time-interval,

the ADAEC shall determine the sender identity to confirm whether the sender is authorized or not. If:

a)	the sender is not the authorized ADAES, the ADAEC shall respond with an HTTP 401 (Unauthorized) response message and avoid the rest of steps; or

b)	the sender is the authorized ADAES and, the ADAEC shall response by HTTP 204 (No Content).

## 6.3	Network slice related application data analytics

### 6.3.1	General

These clauses describe the procedures on the ADAE server and ADAE client side when a consumer such as a VAL server subscribes to the slice-specific application performance analytics. The VAL performance analytics are performed based on ongoing VAL sessions and data from VAL server or database.

## 6.4	Network slice configuration recommendation based on application analytics

### 6.4.1	General

These clauses describe the procedures on the ADAE server and ADAE client side when a consumer such as a VAL server subscribes to the slice usage pattern based on collected data of the slice performance analytics in the past.

## 6.5	Edge load analytics

### 6.5.1	General

These clauses describe the procedures on the ADAE server and ADAE client side when an analytics consumer subscribes to or requests the edge load analytics performed based on data collected from the EDN (EAS and/or EES) and A-ADRF.

## 6.6	UE-to-UE communications performance analytics

### 6.6.1	General

These clauses describe the procedures on the ADAE server and ADAE client side when an analytics consumer such as the VAL server subscribes to the UE-to-UE communications performance analytics performed based on data collected from the ongoing VAL UE-to-UE sessions.

## 6.7	Location accuracy analytics

### 6.7.1	General

These clauses describe the procedures on the ADAE server side when an analytics consumer such as the VAL server subscribes to the location accuracy prediction of a VAL location or a VAL UE based on NWDAF UE mobility analytics or stored data from A-ADRF.

## 6.8	Service application programming interface (API) analytics

### 6.8.1	General

These clauses describe the procedures on the ADAE server side when an analytics consumer such as the VAL server subscribes to the service API analytics.

## 6.9	Application performance analytics

### 6.9.1	General

These clauses describe the procedures on the ADAE server and ADAE server side when the application performance analytics is provided by VAL UEs.

# 7	API Definitions

## 7.1	ADAE_ServiceConfiguration API

### 7.1.1	Introduction

The HTTP URIs used in HTTP protocol for the ADAE service shall have the resource URI structure as defined in clause TBD:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

where:

a)	{apiRoot} shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [y];

b)	<apiName> shall be "adae-sc";

c)	<apiVersion> shall be "v1"; and

d)	The <apiSpecificSuffixes> shall be set as described in clause 7.1.3.

### 7.1.2	Usage of HTTP and common API related aspects

Editor’s note: This clause is FFS

### 7.1.3	Resources

#### 7.1.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 7.1.3.1-1 depicts the resource URI structure of the ADAE_ServiceConfiguration API.

Figure 7.1.3.1-1: Resource URI structure of the ADAE_ServiceConfiguration API

Editor's note: 	Resource URI structure is FFS.

Table 7.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.1.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| Application performance event subscription | /application-performance | POST | Subscription to the VAL performance analytics event |
| UE-to-UE session performance analytics | /ue2ue-session-performance |  |  |
|  | /ue2ue-session-performance/fetch | POST | Request for the UE-to-UE session performance analytics |
| Edge load event subscription | /edge-load | POST | Subscription to the edge load data collection event |
| Service experience | /service-experience | POST | Configure triggers for reports on the service experience information |
|  | / service-experience/pull | POST | Pull a service experience report |

#### 7.1.3.2	Resource: Application performance event subscription

##### 7.1.3.2.1	Description

Application performance event subscription is used by the ADAES to subscribe to the ADAEC for the event of the VAL performance analytics.

##### 7.1.3.2.2	Resource definition

Resource URI: {apiRoot}/adae-sc/<apiVersion>/application-performance

This resource shall support the resource URI variables defined in the table 7.1.3.2.2-1.

Table 7.1.3.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.2.4 in 3GPP TS 29.122 [X] |

##### 7.1.3.2.3	Resource standard methods

###### 7.1.3.2.3.1	POST

This operation is for subscription to the VAL application performance analytics and shall support the URI query parameters specified in table 7.1.3.2.3.1-1.

Table 7.1.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 7.1.3.2.3.1-2and the response data structures and response codes specified in table 7.1.3.2.3.1-3.

Table 7.1.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AppPerfSubs | M | 1 | Subscription to the VAL performance analytics event |

Table 7.1.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AppPerfSubs |  |  | 201 (Created) | Subscription to the VAL performance analytics is created. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [X] shall also apply. |  |  |  |  |

Table 7.1.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/adae-sc/<apiVersion>/application-performance |

##### 7.1.3.2.4	Resource custom operations

None.

#### 7.1.3.3	Resource: UE-to-UE session performance analytics

##### 7.1.3.3.1	Description

This resource is used by the ADAES to request the ADAEC for the UE-to-UE session performance analytics.

##### 7.1.3.3.2	Resource definition

Resource URI: {apiRoot}/adae-sc/<apiVersion>/ue2ue-session-performance

This resource shall support the resource URI variables defined in the table 7.1.3.3.2-1.

Table 7.1.3.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.2.4 in 3GPP TS 29.122 [X] |

##### 7.1.3.3.3	Resource standard methods

None

##### 7.1.3.3.4	Resource custom operations

###### 7.1.3.3.4.1	Overview

Table 7.1.3.3.4.1-1: Custom operations

| Operation name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| Fetch | /ue2ue-session-performance/fetch | POST | Request for the UE-to-UE session performance analytics |

###### 7.1.3.3.4.2	POST

This method is for the ADAES to request the ADAEC the UE-to-UE session performance analytics and shall support the URI query parameters specified in table 7.1.3.3.4.2-1.

Table 7.1.3.3.4.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 7.1.3.3.4.2-2 and the response data structures and response codes specified in table 7.1.3.3.4.2-3.

Table 7.1.3.3.4.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| Ue2UePerfReq | M | 1 | ADAES requests ADAEC for the UE-to-UE session performance analytics |

Table 7.1.3.3.4.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| Ue2UePerfResp | M | 1 | 200 (OK) | ADAEC responses ADAES the UE-to-UE session performance analytics |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [X] shall also apply. |  |  |  |  |

#### 7.1.3.4	Resource: Edge load event subscription

##### 7.1.3.4.1	Description

Edge load data subscription is used by the ADAES to subscribe to the ADAEC for the event of the edge load data collection.

##### 7.1.3.4.2	Resource definition

Resource URI: {apiRoot}/adae-sc/<apiVersion>/edge-load

This resource shall support the resource URI variables defined in the table 7.1.3.4.2-1.

Table 7.1.3.4.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.2.4 in 3GPP TS 29.122 [X] |

##### 7.1.3.4.3	Resource standard methods

###### 7.1.3.4.3.1	POST

This method is the ADAES to subscribe to the ADAEC for the event of the edge-load data collection and shall support the URI query parameters specified in table 7.1.3.4.3.1-1.

Table 7.1.3.4.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 7.1.3.4.3.1-2 and the response data structures and response codes specified in table 7.1.3.4.3.1-3.

Table 7.1.3.4.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EdgeDataSubs | M | 1 | Subscription to the edge load data collection event |

Table 7.1.3.4.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| EdgeDataSubs |  |  | 201 (Created) | Subscription to the edge load data collection is created. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [X] shall also apply. |  |  |  |  |

Table 7.1.3.4.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String' | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/adae-sc/<apiVersion>/edge-load |

#### 7.1.3.5	Resource: Service experience

##### 7.1.3.5.1	Description

The resource is used:

a)	by the ADAES to configure the ADAEC with triggers for reporting service experience information;

b)	by the ADAES to configure the ADAEC to push the service experience information report to the ADAES; and

c)	by the ADAES to pull the service experience information report from the ADAEC.

##### 7.1.3.5.2	Resource definition

Resource URI: {apiRoot}/adae-sc/<apiVersion>/service-experience

This resource shall support the resource URI variables defined in the table 7.1.3.5.2-1.

Table 7.1.3.5.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 5.2.4 in 3GPP TS 29.122 [X] |

##### 7.1.3.5.3	Resource standard methods

###### 7.1.3.5.3.1	POST

This operation is used by the ADAES to configure ADAEC with triggers for reporting the service experience and shall support the URI query parameters specified in table 7.1.3.5.3.1-1.

Table 7.1.3.5.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 7.1.3.5.3.1-2 and the response data structures and response codes specified in table 7.1.3.5.3.1-3.

Table 7.1.3.5.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ConfigRepTrigger | M | 1 | Configure triggers for service experience reporting |

Table 7.1.3.5.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ConfigRepTrigger |  |  | 201 (Created) | Configuration of service experience reporting was successful. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [X] shall also apply. |  |  |  |  |

Table 7.1.3.5.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/adae-scadea-sc/<apiVersion>/service-experience |

##### 7.1.3.5.4	Resource custom operations

###### 7.1.3.5.4.1	Overview

Table 7.1.3.5.4.1-1: Custom operations

| Operation name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| Push | /service-experience/push | POST | Push a service experience report |
| Pull | /service-experience/pull | POST | Pull a service experience report |

###### 7.1.3.5.4.2	Operation: POST

This method is used by the ADAES to configure the ADAEC to push the service experience information report to the ADAES and shall support the URI query parameters specified in table 7.1.3.5.4.2-1.

Table 7.1.3.5.4.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 7.1.3.5.4.2-2 and the response data structures and response codes specified in table 7.1.3.5.4.2-3.

Table 7.1.3.5.4.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| Uri | M | 1 | Configure to report the service experience information |

Table 7.1.3.5.4.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 (No Content) | Configuration to report on the service experience information is accepted. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [X] shall also apply. |  |  |  |  |

###### 7.1.3.5.4.3	Operation: POST

This operation is used by the ADAES to pull the service experience information report from the ADAEC and shall support the URI query parameters specified in table 7.1.3.5.4.3-1.

Table 7.1.3.5.4.3-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 7.1.3.5.4.3-2 and the response data structures and response codes specified in table 7.1.3.5.4.3-3.

Table 7.1.3.5.4.3-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| PullSrvExpInfo | M |  | Request for the report on the service experience |

Table 7.1.3.5.4.3-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SrvExpInfoRep | M | 1 | 200 (OK) | Successfully obtaining the report on the service experience information |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [X] shall also apply. |  |  |  |  |

#### 7.1.4.2	Application performance event notification

##### 7.1.4.2.1	Description

Application performance event notification is by the ADAEC to notify the ADAES, the VAL performance analytics.

##### 7.1.4.2.2	Notification definition

The POST method shall be used for the event notification and the callback URI shall be the one provided by the consumer during the subscription to the event.

Callback URI: {notificationUri}

This method shall support the URI query parameters specified in table 7.1.4.2.2-1.

Table 7.1.4.2.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

If the notification is on the VAL performance analytics, this method shall support the request data structures specified in table 7.1.4.2.2-2 and the response data structures and response codes specified in table 7.1.4.2.2-3.

Table 7.1.4.2.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AppPerfNotif | M | 1 | Notification information of the VAL performance analytics. |

Table 7.1.4.2.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 (No Content) | Notification for the VAL performance analytics event is accepted. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [X] shall also apply. |  |  |  |  |

#### 7.1.4.3	Edge load event notification

##### 7.1.4.3.1	Description

The edge load event notification is used by the ADAEC to notify the ADAES, the edge load data collection.

##### 7.1.4.3.2	Notification definition

The POST method shall be used for the event notification and the callback URI shall be the one provided by the consumer during the subscription to the event.

Callback URI: {notificationUri}

This method shall support the URI query parameters specified in table 7.1.4.3.2-1.

Table 7.1.4.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

If the notification is on the edge load data collection, this method shall support the request data structures specified in table 7.1.4.3.2-2 and the response data structures and response codes specified in table 7.1.4.3.2-3.

Table 7.1.4.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EdgeDataNotif | M | 1 | Notification information of edge load data collection event |

Table 7.1.4.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 (No Content) | Notification for the edge load data collection event is accepted. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [X] shall also apply. |  |  |  |  |

#### 7.1.4.4	Service experience report event notification

##### 7.1.4.4.1	Description

The service experience report event notification is used by the ADAEC to notify the ADAES, the service experience information.

##### 7.1.4.4.2	Notification definition

The POST method shall be used for the event notification and the callback URI shall be the one provided by the consumer during the subscription to the event.

Callback URI: {notificationUri}

This method shall support the URI query parameters specified in table 7.1.4.4.2-1.

Table 7.1.4.4.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

If the notification is on the service experience information, this method shall support the request data structures specified in table 7.1.4.4.2-2 and the response data structures and response codes specified in table 7.1.4.4.2-3.

Table 7.1.4.4.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EdgeDataNotif | M | 1 | Notification of service experience report information |

Table 7.1.4.4.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 (No Content) | Notification of the service experience report information is accepted. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [X] shall also apply. |  |  |  |  |

### 7.1.4	Notifications

#### 7.1.4.1	General

Table 7.1.4.1-1: Notifications overview

| Notification | Callback URI | HTTP method | Description (service operation) |
| --- | --- | --- | --- |
| Application performance event notification | {notificationUri} | POST | Notification for the VAL performance analytics event |
| Edge load event notification | {notificationUri} | POST | Notification for the edge load data collection event |
| Service experience report event notification | {notificationUri} | POST | Notification for the service experience report event |

### 7.1.5	Data model

#### 7.1.5.1	General

This clause specifies the application data model supported by the API.

Table 7.1.5.1-1 specifies the data types defined for the ADAE_ServiceConfiguration API.

Table 7.1.5.1-1: ADAE_ServiceConfiguration API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| Ue2UePerfReq | 7.1.5.2.2 | Request for the UE-to-UE session performance analytics |  |
| Ue2UePerfResp | 7.1.5.2.3 | Response for the UE-to-UE session performance analytics |  |
| ConfigRepTrigger | 7.1.5.2.4 | Configure triggers for reports on the service experience information |  |
| PushSrvExpInfo | 7.1.5.2.5 | Push an individual service experience information |  |
| PullSrvExpInfo | 7.1.5.2.6 | Pull an individual service experience information |  |
| SrvExpInfoRep | 7.1.5.2.7 | Response to pull an individual service experience information |  |

Table 7.1.5.1-2 specifies data types re-used by the ADAE_ServiceConfiguration API service.

Table 7.1.5.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| DurationSec | 3GPP TS 29.571 [X] | Represent the time interval between successive location reports. |  |
| LocationArea | 3GPP TS 29.122 [X] | Represents location information. |  |
| Pc5QoSPara | 3GPP TS 29.571 [X] | Represents policy data on the PC5 QoS parameters. |  |
| ReportingInformation | 3GPP TS 29.523 [X] | Indicates the reporting requirement. |  |
| ValTargetUe | 3GPP TS 29.549 [X] | Used to indicate either VAL User ID or VAL UE ID. |  |
| Uri | 3GPP TS 29.571 [X] | Used to indicate the notification URI. |  |

Editor's note: Data types AppPerfSubs, AppPerfNotif, EdgeDataSubs, and EdgeDataNotif are for FFS.

Editor's note: References need to be added in the reference clause.

#### 7.1.5.2	Structured data types

##### 7.1.5.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 7.1.5.2.2	Type: Ue2UePerfReq

Table 7.1.5.2.2-1: Definition of type Ue2UePerfReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| serverId | string | M | 1 | Identity of the ADAES |  |
| analyticsId | string | M | 1 | Identity of the UE-to-UE session analytics |  |
| valUeIds | array(ValTargetUe) | M | 1..N | One or more VAL UEs, for which the UE-to-UE session performance analytics, is requested. |  |
| pc5Qos | Pc5QoSPara | M | 1 | The QoS attributes to be analyzed at the ADAEC during UE-to-UE session. |  |
| reportConfig | RepInfo | O | 0..1 | The configuration of UE-to-UE session performance analytics reporting. |  |
| area | LocationArea | O | 0..1 | The geographical or service area, for which the UE-to-UE session peformance analytics is requested. |  |
| timeInterval | DurationSec | O | 0..1 | The time interval as the start time point and the end time point, for which the UE-to-UE session performance analytics is applied. |  |

##### 7.1.5.2.3	Type: Ue2UePerfResp

Table 7.1.5.2.-1: Definition of type Ue2UePerfResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dataOutputs | array(string) | M | 1..N | UE-to-UE session performance analytics for prediction or statistics depending on the type and on the requested QoS parameter based on the analytics event. |  |
| valUeIds | array(ValTargetUe) | M | 1..N | One or more VAL UEs, for which the UE-to-UE session performance analytics applies. |  |
| analyticsId | string | M | 1 | Identity of the UE-to-UE session analytics |  |

##### 7.1.5.2.4	Type: ConfigRepTrigger

Table 7.1.5.2.4-1: Definition of type ConfRepTrigger

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valServerId | string | M | 1 | Identity of the VAL server, requested for application performance analytics |  |
| valServerIds | array(string) | M | 1..N | Identify the list of one or more VAL servers for which the configuration of the service experience report applies. |  |
| triggCrit | SrvExpRepCrit | M | 1 | Information criteria about the triggers on which the service experience is to be reported for the VAL server. |  |
| commonTriggCrit | SrvExpRepCrit | O | 0..1 | Information criteria about the triggers (applicable to all VAL servers) on which the service experience is fetched. |  |
| srvExpMeas | DurationSec | O | 0..1 | Information about the service experience measurements which needs to be fetched and included in the report. If not present, by default end-to-end response time is measured. |  |
| NotifyTarget | string | O | 0..1 | The target address which is notified. |  |

Editor's note: 	Data type SrvExpRepCrit is FFS.

##### 7.1.5.2.5	Type: PushSrvExpInfo

Table 7.1.5.2.5-1: Definition of type PushSrvExpInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valUeId | ValTargetUe | M | 1 | Identity of VAL UE |  |
| valServerId | string | M | 1 | Identity of the VAL server, for which the service experience report is sent. |  |
| valServiceId | string | O | 0..1 | Identity of the VAL service |  |
| timeStamp | DurationSec | O | 0..1 | Timestamp as start time and end time of the collected report |  |
| valSrvExpRep | ReportingInformation | O | 0..1 | Report on the VAL service experience |  |

##### 7.1.5.2.6	Type: PullSrvExpInfo

Table 7.1.5.2.6-1: Definition of type PullSrvExpInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valServerId | string | M | 1 | Identity of the VAL server, for which the service experience report is requested. |  |
| valServiceId | string | O | 0..1 | Identity of the VAL service |  |

##### 7.1.5.2.7	Type: SrvExpInfoRep

Table 7.1.5.2.7-1: Definition of type SrvExpInfoRep

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valUeId | ValTargetUe | M | 1 | Identity of VAL UE |  |
| valServerId | string | M | 1 | Identity of the VAL server, for which the service experience report is requested. |  |
| valServiceId | string | O | 0..1 | Identity of the VAL service |  |
| timeStamp | DurationSec | O | 0..1 | Timestamp as start time and end time of the collected report |  |
| valSrvExpRep | ReportingInformation | O | 0..1 | Report on the VAL service experience |  |

### 7.1.6	Error Handling

#### 7.1.6.1	General

HTTP error handling shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [x].

In addition, the requirements in the following clauses shall apply.

#### 7.1.6.2	Protocol Errors

In this release of the specification, there are no additional protocol errors applicable for the ADAE_ServiceConfiguration API.

#### 7.1.6.3	Application Errors

The application errors defined for ADAE_ServiceConfiguration API are listed in table 7.1.6.3-1.

Table 7.1.6.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

### 7.1.7	Feature Negotiation

General feature negotiation procedures are defined in clause 5.2.7 of 3GPP TS 29.122 [x]. Table 7.1.7-1 lists the supported features for ADAE_ServiceConfiguration API.

Table 7.1.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

Annex <A> (normative):
HTTP resource representation and encoding

Annex <B> (Normative):
CoAP resource representation and encoding

Annex <C> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-02 | CT1#140 | C1-230160 |  |  |  | Skeleton for Application Data Analytics Enablement Service | 0.0.0 |
| 2023-05 | CT1#142 | C1-233310 |  |  |  | Application data analytics enablement service abbreviations and refences | 0.1.0 |
| 2023-05 | CT1#142 | C1-234031 |  |  |  | Application data analytics enablement service functional entities | 0.1.0 |
| 2023-05 | CT1#142 | C1-234032 |  |  |  | Application data analytics enablement service procedures | 0.1.0 |
| 2023-05 | CT1#142 | C1-234033 |  |  |  | Application data analytics enablement service procedures | 0.1.0 |
| 2023-10 | CT1#144 | C1-237009 |  |  |  | Correction of general description | 0.2.0 |
| 2023-10 | CT1#144 | C1-237962 |  |  |  | ADAES configuration API | 0.2.0 |
| 2023-10 | CT1#144 | C1-237963 |  |  |  | Application performance analytics configuration API | 0.2.0 |
| 2023-10 | CT1#144 | C1-237964 |  |  |  | Procedure for application performance analytics | 0.2.0 |
| 2023-11 | CT1#145 | C1-239589 |  |  |  | Resource review for ADAE services | 0.3.0 |
| 2023-11 | CT1#145 | C1-239666 |  |  |  | Application performance event subscription for ADAE services | 0.3.0 |
| 2023-11 | CT1#145 | C1-239672 |  |  |  | UE-to-UE session performance for ADAE services | 0.3.0 |
| 2023-11 | CT1#145 | C1-239673 |  |  |  | Edge load event subscription for ADAE services | 0.3.0 |
| 2023-11 | CT1#145 | C1-239588 |  |  |  | Service experience for ADAE services | 0.3.0 |
| 2023-11 | CT1#145 | C1-239674 |  |  |  | Application performance event notification for ADAE services | 0.3.0 |
| 2023-11 | CT1#145 | C1-239675 |  |  |  | Edge load event notification for ADAE services | 0.3.0 |
| 2023-11 | CT1#145 | C1-239586 |  |  |  | Data model for ADAE services | 0.3.0 |
| 2023-11 | CT1#145 | C1-239587 |  |  |  | Error handling for ADAE services | 0.3.0 |
| 2023-11 | CT1#145 | C1-239477 |  |  |  | Notification overview for ADAE services | 0.3.0 |