| 3GPP TS 26.565 V0.2.0 (2022-11) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Split Rendering Media Service Enabler; (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	6

4	General	7

4.1	Overview	7

4.2	Typical Use Cases	7

5	Reference Architecture and Procedures	7

5.1	Reference Architecture	7

5.2	Procedures and Call Flows	7

6	Prerequisites	7

7	Split Rendering Client Functionality	7

8	Split Rendering Control Plane Functionality	7

9	Split Rendering User Plane Functionality	7

9.1	Split Rendering Signalling Protocols	7

9.2	Split Rendering Formats	7

9.3	Split Rendering Transport Protocols	7

Annex A	 (Informative): Implementation Guidelines	8

Annex A.1 Guidelines for Application Developers	8

Annex A.2 Guidelines for Split Rendering MSE Implementers	8

Annex A.3 Conformance Testing	8

Annex B (normative): IDL Definition of Client API	8

Annex X (informative): Change history	9

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

This specification defines a media service enabler for split rendering in the 5G system.

# 1	Scope

The present document defines a Media Service Enabler for Split Rendering according to the guidelines of TR26.857 [1].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 26.857: "5G Media Service Enablers".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	General

## 4.1	Overview

## 4.2	Typical Use Cases

# 5	Reference Architecture and Procedures

Editor’s Note: The terminalogy and reference-point naming convention needs to be aligned in  this document.

## 5.1	Reference Architecture

### 5.1.1	Introduction

We define different variants of the reference architecture for the split rendering MSE, each representing a different perspective and level of detail.

### 5.1.2 	Client Architecture

Figure 5.1.2-1 depicts the client architecture.

Figure 5.1.2-1 – Device architecture of AR UE

### 5.1.3	End-to-End Architecture

The end-to-end architecture is depicted in figure 5.1.3-1.

Editor’s Note: this architecture variant is TBD based on the MeCAR discussions.

### 5.1.4 Control Architecture

Figure 5.1-3 – Split management architecture

### 5.1.5	User Plane Architecture

Figure 5.1.5-1 depicts the user plane architecture for split rendering.

## 5.2	Procedures and Call Flows

### 5.2.1 	Call flow for Split Rendering instance discovery

#### 5.2.1.1	Client-driven procedures and call flows

Figure 5.2.1-1 demonstrates a call flow for setting up the split rendering by the client.

Figure 5.2.1-1: High-level call flow for initiating a split

Steps:

- The 5GMS Application Service Provider requests the SRF the provisioning a split management session.

- The split management session is announced to the Application as part of the Service Access Information.

- The 5GMS-Aware application requests a split of the client media functions from the SRC.

- The SRC inquires the Media Service Client to discover the client’s media capabilities.

- The SRC requests the SRF split of the client media functions.

- The SRF inquires the 5GMS AS about its capabilities.

- The SRC and SRF negotiate on the acceptable capabilities for the device and agree on the split option.

- The SRF requests the 5GMS AS to start the split-rendering process on the edge.

- The SRF acknowledges the SRC that the split-rendering on edge is running and provides its access information.

- The SRC acknowledges the Application that the split-rendering on edge is running.

- The 5GMS-aware Application request starting a media session from the 5GMS client.

- The 5GMS client connects to the split-rendering on the 5GMS AS using provided information in step 9.

#### 5.2.1.2	Network-driven procedures and call flows

Figure 5.2.1.2-1 demonstrates a call flow for setting up the split rendering by the client.

Figure 5.2.1.2-1: High-level call flow for the network-driven split management

Steps:

- The 5GMS Application Service Provider requests the SRF the provisioning a split management session.

- The split management session is announced to the Application as part of the Service Access Information.

- The SRF inquires the 5GMS AS about its capabilities.

- The SRF requests the SRC split of the client media functions.

- The SRC inquires the Media Service Client to discover the client’s media capabilities.

- The SRC and SRF negotiate on the acceptable capabilities for the device and agree on the split option.

- The SRF requests the 5GMS AS to start the split-rendering process on the edge.

- The SRF acknowledges the SRC that the split-rendering on edge is running and provides its access information.

- The SRC acknowledges the Application that the split-rendering on edge is running.

- The 5GMS-aware Application request starting a media session from the 5GMS client.

- The 5GMS client connects to the split-rendering on the 5GMS AS using provided information in step 9.

### 5.2.2 	Call flow for Split Rendering session setup

The split rendering operation can be described by the following call flow:

The steps are:

- The Presentation Engine discovers the split rendering EAS and sets up a connection to it. It provides information about its rendering capabilities and the OpenXR configuration.

- In response, the split rendering EAS creates a description of the split rendering output and the input it expects to receive from the UE.

- The Presentation Engine requests the buffer streams from the MAF, which in turn establishes a connection to the split rendering EAS to stream pose and retrieve split rendering buffers.

- The Source Manager retrieves pose and user input from the XR runtime.

- The Source Manager shares the pose predictions and user input actions with the split rendering EAS

- The split rendering EAS uses that information to render the frame

- The rendered frame is encoded and streamed down to the MAF

# 6	Prerequisites

## 6.1	Requirements on 5G System

## 6.2	Requirements on Device APIs and Functionality

MSE-7

# 7	Split Rendering Application Function

## 7.1	Functionality

## 7.2	RESTful APIs

MSE-5, MSE-1 (optional)

# 8	Split Rendering User Plane

MSE-4

## 8.1	Split Rendering Signalling Protocols

## 8.2	Split Rendering Formats

### 8.2.1 	General

### 8.2.2 	Pixel Streaming Profile

#### 8.2.2.1 	Overview

The full-prerendering profile is restricted to the support of 2D content exclusively. The capabilities of the receiving UE are shared with the split rendering server prior to the start of the split rendering session. These capabilities and configurations would indicate the audio-visual output setup on the UE. For example, it would indicate that the output device is an HMD that supports 2 views and stereo audio.

#### 8.2.2.2 	Downlink Formats

The visual formats supported in this profile match the options that are supported by OpenXR. The supported view configurations are:

- Mono: a single view

- Stereo: one view per eye

The following composition layers are supported:

- Projection: projection of the scene to a 2D plane using a perspective camera

- Quad: a 2d surface that is composed in the 3D space by the XR runtime

- Equirectangular: an equirectangular projection of the 3D space that is usually used to provide a background

- Cubemap: a set of 6 swapchain images that represent a projection of the 3D scene onto a cube

Each swapchain image will have the following properties:

- Format: RGB, RGB with Alpha (RGBA), and single-channel Depth formats with different precisions. RGB may be recovered from the coded YUV video stream. Depth information may be coded a separate video stream.

- Dimension: width and height of the swapchain image

- Mipmap: count of the level of detail of the swapchain image. The swapchain images maybe created at the UE side. Some Graphics Engines expect that the image dimensions are a power of 2.

For audio, the following formats are to be supported:

- Stereo audio mixed and binauralized based on the viewer’s current pose

- HOA audio mixed based on the viewer’s current position that extracted from the pose

#### 8.2.2.3 	Uplink Formats

The rendering process relies on the reception of pose predictions and user input. The pose information is formatted as follows:

- An array of multiple pose predictions

- Each pose prediction consists of a position and orientation component as a 3D (coordinates) and 4D (quaternion) vectors respectively.

- The prediction timestamp associated with the predicted pose

- An XR space for which the pose is created. If not present, this defaults to the viewer’s XR space.

The user input may be described following OpenXR’s convention on naming the actions.

## 8.3	Split Rendering Transport Protocols

# 9	Split Rendering Client

## 9.1	Functionality

Reference pre-requisites, user plane functionality, control plane

## 9.2	Client API

MSE-6

Annex A	 (Informative):
Implementation Guidelines

Annex A.1 Guidelines for Application Developers

Annex A.2 Guidelines for Split Rendering MSE Implementers

Annex A.3 Conformance Testing

Annex B (normative):
IDL Definition of Client API

Annex X (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |