| 3GPP TR 26.804 V17.0.0 (2022-03) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on 5G media streaming extensions (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	10

3.1	Terms	10

3.2	Symbols	10

3.3	Abbreviations	10

5	Key Topics	11

5.1	Introduction	11

5.2	Content Preparation	11

5.2.1	Overview	11

5.2.2	Gap Analysis of 26.512	12

5.2.3	Use-cases	12

5.2.3.1	Basic CMAF/DASH/HLS multi-rate live streaming of user-generated content	12

5.2.4	Collaboration Scenarios	14

5.2.4.1	Content preparation before downlink streaming	14

5.2.4.2	Content preparation after uplink ingest streaming	14

5.2.4.3	Content preparation between uplink ingest and downlink streaming	15

5.2.5	Deployment Architectures	16

5.2.6	Mapping to 5G Media Streaming and High-Level Call Flows	17

5.2.6.1	Call flow for content preparation before downlink streaming	17

5.2.6.2	Call flow for content preparation after uplink streaming	20

5.2.6.3	Baseline call flow for content processing between uplink streaming and downlink streaming	22

5.2.7	Potential open issues	23

5.2.7.1	Open issues in collaboration scenario 1: Content preparation before downlink streaming	23

5.2.7.2	Open issues in collaboration scenario 2: content preparation after uplink streaming	25

5.2.7.3	Open issues in collaboration scenario 3: content preparation between uplink and downlink	25

5.2.8	Candidate Solutions	25

5.2.8.1	Content Preparation Template requirements	25

5.2.8.1.1	Unencrypted single CMAF track to single unencrypted CMAF switching set	25

5.2.8.2	Content Preparation Template candidates	26

5.2.8.2.1	CMAF input format candidate 1: DASH MPD manifest	26

5.2.8.2.2	CMAF input format candidate 2: A new document format	26

5.2.8.2.3	CMAF output format candidate 3: Extended manifest format	26

5.2.8.2.4	CMAF output format candidate 4: Manifest with supplementary encoding parameters document	26

5.2.8.2.5	CMAF output format candidate 5: A document defining both the output manifest and encoding parameters	27

5.2.8.3	Combining the Content Preparation Template candidate solutions	27

5.2.8.4	Combined CMAF input and output formats candidate: NBMP Workflow Description Document	28

5.2.8.5	Address translation for complex pull requests	29

5.2.9	Conclusion and recommendations	30

5.3	Traffic Identification	30

5.3.1	Description	30

5.3.2	Collaboration Scenarios	33

5.3.2.1	General Collaboration Scenarios	33

5.3.2.2	Differentiated Services/ToS-enabled Collaboration Scenarios	34

5.3.3	Deployment Architectures	35

5.3.4	Mapping to 5G Media Streaming and High-Level Call Flows	36

5.3.4.1	General	36

5.3.4.2	Usage of 5-tuples for Traffic Identification	36

5.3.4.3	Usage of ToS Traffic Class for Traffic Identification	38

5.3.4.4	Usage of Packet Flow Descriptions for Traffic Identification	39

5.3.5	Potential open issues	40

5.3.6	Candidate Solutions	41

5.3.6.1	Overview	41

5.3.6.2	Candidate IP-PFS Solution 1: Using IP ToS marking for downlink-only QoS flow mapping	41

5.3.6.3	Candidate IP-PFS Solution 2: Using IP ToS marking for uplink-only QoS flow mapping	41

5.3.6.4	Candidate IP-PFS Solution 3a: Using IP ToS marking for bi-directional QoS flow mapping, initiated by downlink traffic	42

5.3.6.5	Candidate IP-PFS Solution 3b: Using IP ToS marking for bi-directional QoS flow mapping, initiated by downlink traffic	44

5.3.6.6	Candidate IP-PFS Solution 4a: Using ToS marking for bi-directional QoS flow mapping, initiated by uplink traffic	46

5.3.6.7	Candidate IP-PFS Solution 4b: Using ToS marking for bi-directional QoS flow mapping, initiated by uplink traffic	48

5.3.6.7	Candidate IP-PFS Solution 4c: Using ToS marking for bi-directional QoS flow mapping, initiated by uplink traffic	50

5.3.7	Conclusions	52

5.4	Additional/new transport protocols	52

5.4.1	Description	52

5.4.1.1	General	52

5.4.1.2	Performance Considerations for HTTP/3 over 5G Networks	53

5.4.1.3	Performance Considerations for IETF QUIC over 5G networks	54

5.4.1.4	Management Considerations for HTTP/3 in 5G networks	54

5.4.1.5	HTTP/3 client operation with an HTTP/3 server	55

5.4.1.6	QLOG metrics reporting for HTTP/3 and QUIC	55

5.4.2	Collaboration Scenarios	56

5.4.2.1	General	56

5.4.2.2	HTTP/3 collaboration for downlink media streaming	56

5.4.2.3	Collaboration for uplink media streaming using HTTP/3	56

5.4.3	Deployment Architectures	57

5.4.3.1	General	57

5.4.3.2	HTTP/3 deployment in downlink media streaming	57

5.4.3.3	HTTP/3 deployment in uplink media streaming	58

5.4.4	Mapping to 5G Media Streaming and High-Level Call Flows	58

5.4.4.1	General	58

5.4.5	Potential open issues	58

5.4.5.1	Introduction	58

5.4.5.2	Streaming Protocols taking advantage of HTTP/3 capabilities	58

5.4.5.3	3GPP-specific impediments to HTTP/3 deployment	58

5.4.5.4	Adaptive Streaming clients operating on top of HTTP/3 capabilities	59

5.4.5.5	5GMS Operation taking advantage of HTTP/3 capabilities	59

5.4.5.6	Addition of HTTP/3 as a supported HTTP version in 3GPP specifications	60

5.4.5.7	Pace of HTTP/3 deployment in 3GPP environments	60

5.4.6	Candidate Solutions	60

5.4.6.1	Metrics reporting using QLOG events	60

5.5	Uplink media streaming	61

5.5.1	Description	61

5.5.1.1	Overview	61

5.5.1.2	Gap analysis of TS 26.501	61

5.5.1.3	Gap analysis of TS 26.512	62

5.5.1.4	Gap analysis between TS 26.238 (FLUS) and TS 26.512 (5G Media Streaming)	62

5.5.2	Collaboration Scenarios	63

5.5.2.1	Overview	63

5.5.2.2	Collaboration Scenario 1	63

5.5.2.3	Collaboration Scenario 2	64

5.5.2.4	Collaboration Scenario 3	64

5.5.2.5	Collaboration Scenario 4	65

5.5.2.6	Collaboration Scenario 5	65

5.5.2.7	Collaboration Scenario 6	66

5.5.3	Deployment Architectures	66

5.5.4	Mapping to 5G Media Streaming and High-Level Call Flows	67

5.5.4.1	Collaboration scenario 1 call flow	67

5.5.4.2	Collaboration scenario 2 call flow	68

5.5.4.3	Collaboration scenario 3 call flow	69

5.5.4.4	Collaboration scenario 4 call flow	71

5.5.4.5	Collaboration scenario 5 call flow	73

5.5.4.6	Collaboration scenario 6 call flow	74

5.5.5	Potential open issues	74

5.5.5.1	Potential open issues in 5G Media Streaming stage 3	74

5.5.5.2	Potential open issues compared with FLUS	75

5.5.5.2.1	General	75

5.5.5.2.2	Discussion 1	75

5.5.5.2.3	Discussion 2	76

5.5.5.2.4	Discussion 3	76

5.5.6	Candidate Solutions	76

5.5.6.1	Content egest protocols	76

5.5.6.2 	Content Publishing Configuration API	77

5.5.6.3 	Content Publishing Configuration Template	77

5.5.6.4	Uplink entry point	78

5.5.7	Conclusion	79

5.6	Background traffic	80

5.6.1	Description	80

5.6.2	Collaboration Scenarios	80

5.6.3	Deployment Architectures	80

5.6.4	Mapping to 5G Media Streaming and High-Level Call Flows	81

5.6.5	Potential open issues	81

5.6.6	Candidate Solutions	81

5.6.6.1	Existing APIs to provision Background Data Transfer	81

5.6.6.2	Potential Solution	82

5.6.7	Conclusion	83

5.7	Content-Aware Streaming	83

5.7.1	Description	83

5.7.2	Collaboration Scenarios	85

5.7.3	Deployment Architectures	86

5.7.4	Mapping to 5G Media Streaming and High-Level Call Flows	86

5.7.5	Potential open issues	86

5.7.6	Candidate Solutions	87

5.8	Network Event usage	87

5.8.1	Description	87

5.8.1.1	Events exposed by 5GMS AF	87

5.8.1.2	Events consumed by 5GMS AF	87

5.8.2	Collaboration Scenarios	88

5.8.3	Deployment Architectures	88

5.8.4	Mapping to 5G Media Streaming and High-Level Call Flows	89

5.8.5	Potential open issues	90

5.8.6	Candidate Solution	90

5.8.6.1	UE Data Collection via Direct and Indirect Methods	90

5.8.6.2	AF Collection of CDN Access Logs	92

5.8.6.3	 Candidate media-related information for Event Exposure	92

5.9	Per-application-authorization	93

5.9.1	Description	93

5.9.1.1	General	93

5.9.2.2	OAuth 2.0	93

5.9.2	Collaboration Scenarios	94

5.9.2.1	Collaboration A: UE hosting multiple Applications	94

5.9.2.2	Collaboration B: Applications with multiple subscription levels	95

5.9.3	Role distribution in the 5GMS deployments	95

5.9.4	Mapping to 5G Media Streaming and High-Level Call Flows	96

5.9.5	Potential open issues	96

5.9.6	Candidate Solutions	96

5.9.6.1	General	97

5.9.6.2	Solution 1: Use of a Callback for authorization	97

5.9.6.3	Solution 2: Time-limited authorization token provisioning	98

5.9.7	Conclusions	99

5.10	Support for encrypted and high-value content	99

5.10.1	Description	99

5.10.2	Collaboration Scenarios	100

5.10.3	Deployment Architectures	101

5.10.4	Mapping to 5G Media Streaming and High-Level Call Flows	101

5.10.5	Potential open issues	101

5.10.6	Candidate Solutions	101

5.11	TV-grade mass distribution of unicast Live Services	101

5.11.1	Description	101

5.11.1.1	General	101

5.11.1.2	Scalability	101

5.11.1.3	Consistent quality	101

5.11.1.4	High bandwidth requirements	103

5.11.1.5	Target latency constraints	103

5.11.1.6	TV Experiences	105

5.11.1.7	Summary of service requirements for Mass grade TV distribution	106

5.11.2	Deployment Architectures	107

5.11.2.1	Distribution of low-latency media streams	107

5.11.2.2	Operation Point – Establishment and Monitoring	107

5.11.3	Collaboration Scenarios	108

5.11.3.1	General	108

5.11.3.2	Distribution of low-latency media streams	108

5.11.4	Mapping to 5G Media Streaming and High-Level Call Flows	109

5.11.4.1	General: Distribution of “Operation Point Services”	109

5.11.4.2	Collaboration 1: MNO provides encoding and packaging	110

5.11.4.3	Collaboration 2: MNO provides DASH distribution	110

5.11.4.4	Collaboration 3: MNO acts as CDN	110

5.11.4.5	Operation Point example	111

5.11.5	Potential open issues	111

5.11.6	Candidate Solutions	112

5.11.7	Conclusions	112

5.12	Network Slicing Extensions for 5G Media Streaming	113

5.12.1	Description	113

5.12.1.1	Overview	113

5.12.1.2	Network Slicing Extensions in SA4	114

5.12.2	Collaboration Scenarios	115

5.12.3	Deployment Architectures	116

5.12.4	Mapping to 5G Media Streaming and High-Level Call Flows	116

5.12.5	Potential open issues	116

5.12.6	Candidate Solutions	116

5.12.6.1	Network slices and Operation Points provisioned at M1	116

5.12.7	Conclusion	118

6	Conclusions	118

6.1	List of Conclusions	118

6.2	Content preparation	118

6.3	Traffic identification	119

6.4	Additional/new transport protocols	119

6.5	Uplink media streaming	119

6.6	Background traffic	119

6.7	Content-aware streaming	120

6.8	Network Event usage	120

6.9	Per-application-authorization	120

6.10	Support for encrypted and high-value content	120

6.11	TV-grade mass distribution of unicast Live Services	120

6.12	Network Slicing Extensions for 5G Media Streaming	121

7	Recommendations	121

Annex A: Media Streaming Protocols	123

A.1	Status and usage of Web protocols	123

A.1.1	M4d protocol usage	123

A.1.2	Results of HTTP protocol version usage study	123

Annex X (informative): Change history	124

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document …

This Technical Report identifies and evaluates a set of potential improvements and extensions, referred to as key topics. The key topics are

-	Content Preparation

-	Traffic Identification

-	Additional / New transport protocols

-	Uplink media streaming

-	Background traffic

-	Content Aware Streaming

-	Network Event usage

-	Per-application-authorization

-	Support for encrypted and high-value content

-	Scalable distribution of unicast Live Services

For each of the above key topics, the following objectives are identified:

1.	Document the above key topics in more detail, in particular how they relate to the 5GMS Architecture and protocols.

2.	Study collaboration scenarios between the 5G System and Application Provider for each of the key topics.

3.	Based on the 5GMS Architecture, develop one or more deployment architectures that address the key topics and the collaboration models.

4.	Map the key topics to basic functions and develop high-level call flows.

5.	Identify the issues that need to be solved.

6.	Provide candidate solutions (including call flows) for each of the identified issues.

7.	Coordinate work with other 3GPP groups e.g. SA2, SA3, SA5, and others as needed.

8.	Coordinate work with external organizations such as DASH-IF, CTA WAVE, ISO/IEC JTC29 WG3 (MPEG Systems), or IETF, as needed.

9.	Identify gaps and recommend potential normative work for stage-2 call flows and possibly stage-3.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	Akamai Blog, "A QUICk Introduction to HTTP/3", April 2020, https://developer.akamai.com/blog/2020/04/14/quick-introduction-http3

[3]	Fielding, R., Nottingham, M., and J. Reschke, "HTTP/1.1", Work in Progress, Internet-Draft, draft-ietf-httpbis-messaging-13, 14 December 2020, http://www.ietf.org/internet-drafts/draft-ietf-httpbis-messaging-13.txt

[4]	Belshe, M., Peon, R., and M. Thomson, Ed., "Hypertext Transfer Protocol Version 2 (HTTP/2)", RFC 7540, May 2015, https://www.rfc-editor.org/info/rfc7540

[5]	draft-ietf-quic-http-34, "Hypertext Transfer Protocol Version 3 (HTTP/3)", February 2021

[6]	D. Bhat, A. Rizk, and M. Zink, "Not so QUIC: A Performance Study of DASH over QUIC," NOSSDAV'17: Proceedings of the 27th Workshop on Network and Operating Systems Support for Digital Audio and VideoJune 2017 Pages 13–18 https://doi.org/10.1145/3083165.3083175

[7]	AWS: "Achieving Great Video Quality Without Breaking the Bank", Streaming Media June 2019, https://pages.awscloud.com/rs/112-TZM-766/images/GEN elemental-wp-achieving-great-video-quality-without-breaking-the-bank.pdf

[8]	Netflix, "Optimized shot-based encodes: Now Streaming!", Netflix Blog, May 2018, https://netflixtechblog.com/optimized-shot-based-encodes-now-streaming-4b9464204830

[9]	DASH-IF/DVB: "Report on Low-Latency Live Service with DASH", July 2017, available here: https://dash-industry-forum.github.io/docs/Report%20on%20Low%20Latency%20DASH.pdf

[10]	DASH-IF: "IOP Guidelines v5, Low-latency Modes for DASH", available here: https://dash-industry-forum.github.io/docs/CR-Low-Latency-Live-r8.pdf

[11]	ISO/IEC 23009-1: "Information technology — Dynamic adaptive streaming over HTTP (DASH) — Part 1: Media presentation description and segment formats".

[12]	IETF RFC 8673: "HTTP Random Access and Live Content".

[13]	3GPP TR 26.939: "Guidelines on the Framework for Live Uplink Streaming (FLUS)".

[14]	3GPP TS 26.238: "Uplink Streaming".

[15]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[16]	3GPP TS 26.512: "5G Media Streaming (5GMS); Protocols".

[17]		ISO/IEC 13818-1:2019: "Information technology — Generic coding of moving pictures and associated audio information — Part 1: Systems".

[18]	SCTE 35 2020: "Digital Program Insertion Cueing Message", https://www.scte.org/pdf-redirect/?url=https://scte-cms-resource-storage.s3.amazonaws.com/SCTE-35-2020_notice-1609861286512.pdf

[19]	ISO/IEC 23000-19:2020: "Information technology — Multimedia application format (MPEG-A) —Part 19: Common media application format (CMAF) for segmented media".

[20]	ISO/IEC 23009-1:2019/DAMD1: "Information technology — Dynamic adaptive streaming over HTTP (DASH) — Part 1: Media presentation description and segment formats — Amendment 1: CMAF support, events processing model and other extensions".

[21]	VSF TR-06-01:2020, "RIST Simple Profile", https://vsf.tv/download/technical_recommendations/VSF_TR-06-1_2020_06_25.pdf

[22]	VSF TR-06-02: "RIST Main Profile", https://www.videoservicesforum.org/download/technical_recommendations/VSF_TR-06-2_2020_03_24.pdf

[23]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[24]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[25]	3GPP TS 29.517: "5G System; Application Function Event Exposure Service; Stage 3".

[26]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane nodes; Stage 3".

[27]	IETF RFC 6733: "Diameter Base Protocol".

[28]	3GPP TS 29.514: "5G System; Policy and Charging Control over Rx reference point; Stage 3".

[29]		IETF RFC 7657: "Differentiated Services (Diffserv) and Real-Time Communication", November 1995.

[30]	IETF RFC 3168: "The Addition of Explicit Congestion Notification (ECN) to IP", September 2001.

[31]	C. Krasic, M. Bishop, and A. Frindell, Ed., draft-ietf-quic-qpack-21, "QPACK: Header Compression for HTTP/3", Work in Progress, Internet-Draft, 2 February 2021.

[32]	IETF RFC 9000: "QUIC: A UDP-Based Multiplexed and Secure Transport", May 2021.

[33]	IETF RFC 9001: "Using TLS to Secure QUIC", May 2021.

[34]	IETF, RFC 9002: "QUIC Loss Detection and Congestion Control", May 2021.

[35]	IETF RFC 5681: "TCP Congestion Control".

[36]	M. Kuehlewind and B. Trammell, draft-ietf-quic-manageability-11, "Manageability of the QUIC Transport Protocol", Work in Progress, Internet-Draft, 30 June 2021.

[37]	N. Cardwell et. al. "BBR Updates: Internal Deployment, Code, Draft Plans", 9 March 2021, https://datatracker.ietf.org/meeting/110/materials/slides-110-iccrg-bbr-updates-00.pdf

[38] 	ETSI TS 103 799: "Publicly Available Specification (PAS); DASH-IF Content Protection Information Exchange Format".

[39]	ISO/IEC JTC1/SC29/WG11/N19062 23090-8 FDIS: "MPEG-I: Network-based Media Processing — Network-Based Media Processing Specification".

[40]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".[41]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[42]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[43]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[44]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[45]	3GPP TS 29.512: "5G System; Session Management Policy Control Service; Stage 3".

[46]	3GPP TS 26.803: "5G Media Streaming (5GMS); Architecture extensions".

[47]	3GPP TS 23.558: "Architecture for enabling Edge Applications (EA)".

[48]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[49]	Tdoc S4-210723: "Generic architecture for data collection and reporting", submission from BBC, Dolby Laboratories Inc., LM Ericsson and Qualcomm Incorporated to SA4#114-e, May 19-28, 2021.

[50]	Tdoc S2-2103267: "Extension of Naf_EventExposure for observed service experience data collection from UEs", CR from InterDigital to SA2#144e, Apr 12-16, 2021.

[51]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

[52]	Tdoc S2-2104496: "Extension of Naf_EventExposure for observed service experience data collection from UEs", CR from Qualcomm Incorporated to SA2#145e, May 17-28, 2021.

[53]	3GPP TS 26.118: "Virtual Reality (VR) profiles for streaming applications".

[54]	3GPP TS 26.346: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[55]	3GPP TS 29.554: "Background Data Transfer Policy Control Service; Stage 3".

[56]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[57]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[58]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[59]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[60]	3GPP TS 28.540: "Management and orchestration; 5G Network Resource Model (NRM); Stage 1".

[61]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[62]	3GPP TS 28.542: "Management and orchestration of networks and network slicing; 5G Core Network (5GC) Network Resource Model (NRM); Stage 1".

[63]	3GPP TS 28.543: "Management and orchestration of networks and network slicing; 5G Core Network (5GC) Network Resource Model (NRM); Stage 2 and stage 3".

[64]	3GPP TS 28.545: "Management and orchestration; Fault Supervision (FS)".

[65]	3GPP TS 28.546: "Management and orchestration of networks and network slicing; Fault Supervision (FS); Stage 2 and stage 3".

[66]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[67]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[68]	3GPP TS 23.434: " Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows ".

[69]	3GPP TS 23.700-99: " Study in Network slice capability exposure for application layer enablement (NSCALE)".

[70]	3GPP TS 29.520: " 5G System; Network Data Analytics Services; Stage 3".

[71]	3GPP TR 23.700-40: "Study on enhancement of network slicing; Phase 2".

[72]	3GPP TS 26.531: “Data Collection and Reporting; General Description and Architecture”.

[73]	3GPP TR 26.802: "Multicast Architecture Enhancement for 5G Media Streaming".

[74] 	IETF RFC 822: "STANDARD FOR THE FORMAT OF ARPA INTERNET TEXT MESSAGES", August 13, 1982.

[75]	IETF RFC 1521: "MIME (Multipurpose Internet Mail Extensions)", September 1993.

[76]	IETF RFC 2474: "Definition of the Differentiated Services Field (DS Field) in the IPv4 and IPv6 Headers".

[77]	IETF RFC 2475: "An Architecture for Differentiated Services".

[78]	IETF RFC 3246: "An Expedited Forwarding PHB (Per-Hop Behavior)".

[79]	IETF RFC 2597: "Assured Forwarding PHB Group".

[80]	S. Hurst, draft-hurst-quic-rtp-tunnelling: "QRT: QUIC RTP Tunnelling", Internet-Draft, Work in Progress.

[81]	J. Ott and M. Engelbart, draft-engelbart-rtp-over-quic: "RTP over QUIC", Internet-Draft, Work in Progress.

[82]	SRT Alliance, “Secure Reliable Transport (SRT) Protocol”, https://github.com/Haivision/srt

[83]	M.P. Sharabayko and M.A. Sharabayko, draft-sharabayko-srt-over-quic-00 ,“Tunnelling SRT over QUIC”, Internet-Draft, Work in Progress, 28 July 2021.

[84]	Robin Marx, Luca Niccolini, Marten Seemann, draft-ietf-quic-qlog-main-schema-01, "Main logging schema for qlog", Internet-Draft, Work in Progress, 25 October 2021.

[85]	Robin Marx, Luca Niccolini, Marten Seemann, draft-ietf-quic-qlog-h3-events-00, "HTTP/3 and QPACK event definitions for qlog", Internet-Draft, Work in Progress, 10 June 2021.

[86]	Robin Marx, Luca Niccolini, Marten Seemann, draft-ietf-quic-qlog-quic-events-00, "QUIC event definitions for qlog", Internet-Draft, Work in Progress, 10 June 2021.

[87]	Roger Pantos and William May, Jr., "HTTP Live Streaming", RFC 8216, August 2017.

[88]	3GPP TR 26.925: "Typical traffic characteristics of media services on 3GPP networks".

[89]	3GPP TR 26.917: "Multimedia Broadcast Multicast Services (MBMS) and Packet-switchedStreaming Service (PSS) enhancements to support television services".

[90]		"DASH-IF WebRTC-based Streaming", https://dashif.org/news/webrtc/

[91]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[92]	IETF RFC 6750: "The OAuth 2.0 Authorization Framework: Bearer Token Usage".

[93]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[94]	3GPP TS 26.531: "Data Collection and Reporting; General Description and Architecture".

[95]	3GPP TS 26.532: "Data Collection and Reporting; Protocols and Formats".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

CDN	Content Delivery Network

DS	Differentiated Service

FAR	Forward Action Rule

MAR	Multi-Access Rule

PDR	Packet Detection Rule

PFCP	Packet Forwarding Control Protocol

QER	QoS Enforcement Rule

QLOG	QUIC Logging

PHB	Per-Hop Behaviour

PFD	Packet Flow Description

SDF	Service Data Flow

URL	Uniform Resource Locator

URR	Usage Reporting Rule

# 5	Key Topics

## 5.1	Introduction

## 5.2	Content Preparation

### 5.2.1	Overview

[TS 26.501 [15] includes the high-level concept of content preparation and/or modification within the 5G Media Streaming System. However, Content Preparation has not yet been defined in detail in TS 26.512 [16]. The extent of content preparation support (including. use- cases, functionalities and features, and resulting formats) needed in the existing 5G Media Streaming Architecture, and how these functions can be realized in with current 5GMS architecture are subject of this study.]

Content preparation may be applied in the following scenarios:

1.	In uplink streaming: on an uplinked stream, providing the resulting content to Application Provider.

2.	In downlink streaming: on an input stream from an Application Provider, providing the resulting content for distribution.

3.	Between uplink and downlink streaming: on an uplink stream, providing the resulting content for distribution.

Two aspects are of relevance for the discussion on Content Preparation:

-	Uplink ingest content formats and protocols

-	Content preparation instructions

On uplink ingest formats, a survey of existing formats is needed to understand the available options. Examples include

1)	MPEG-2 Transport Stream [17] with SCTE metadata [18].

2)	CMAF content [19] with timed metadata tracks.

3)	CMAF content [19] with a manifest such as the MPD [20].

4)	Reliable Internet Stream Transport (RIST) is an open source, open specification transport protocol designed for reliable transmission of video over lossy networks (including the internet) with low latency and high quality. It is currently under development under the Video Services Forum's "RIST Activity Group." To date, two open specifications have been produced:

a.	TR-06-1 (RIST Simple Profile) [21], first released in 2018, defines the basic ARQ (Automatic Repeat Query) technology used to recover lost packets. It has been updated in 2020 to include an optional "RTT Echo" message that streamlines the process of configuring packet buffers.

b.	TR-06-2 (RIST Main Profile) [22], released in 2020, defines additional functions required for commercial applications, including stream encryption, sender/receiver authentication, in-band data tunneling, and bandwidth optimization.

5)	Secure Reliable Transport (SRT) is an open-source video transport protocol maintained by the SRT Alliance [82].

### 5.2.2	Gap Analysis of 26.512

TS 26.512 [16] defines a very limited set of features for content preparation:

1.	The location for content preparation template provisioning.

2.	CRUD Operation for content preparation template.

The current specification neither defines the uplink ingest formats nor the content preparation instructions.

### 5.2.3	Use-cases

#### 5.2.3.1	Basic CMAF/DASH/HLS multi-rate live streaming of user-generated content

Description Kim is subscribed to an Application for live streaming of captured videos from her everyday life. Based on the previous number and diversity of Kim’s usual audience (e.g. close friends), the Application/Application Provider has an “audience codecs-rates” profile which represents the typical number of streams based on Kim’s previous streaming sessions and provides the corresponding Content Preparation Template to the MNO as Kim starts the session live stream session. While Kim is streaming a single stream through her device’s uplink, the MNO processes the received content based on the Content Preparation Template and provides multi-rate tracks for distribution. If new users join Kim’s streaming session which cannot be supported with the current codecs-rates, the Application/Application Provider may update the Content Preparation Template in the session, and the MNO updates the content preparation process accordingly.

Categorization Delivery: Live Streaming Device: Any device connected to the cell network

Preconditions 1. On the device: a. A 3GPP supported encoder is installed. b. UE is 5GMS capable. c. UE’s Application is installed which supports 5GMS. 2. On the network: a. The AS may or may not have resources for running a requested content preparation with the update to the Content Preparation Template and responds accordingly.

Nominal Cost Analysis The cost of service increases linearly with the number of uplink ingest streams. The cost of service increases less than linearly with the number of download streaming clients because the encoding and caching requirement are common to a large number of viewers.

Potential Standardization Status and Needs The Content Preparation Template is expected to define uplink ingest format(s) as well as the following instructions for content preparation: 1. Input characteristics. 2, Outputs characteristics. 3. The media processes and/or functionalities applied. 4. Mechanism for updating the content preparation template. Content preparation for CMAF streaming when the uplink ingest format in CMAF and output is CMAF content with DASH and HLS manifests for live and on-demand services. In this case, the Content Preparation Template should include the following: 1. Uplink ingest characteristics such as: a. Video characteristics b. Audio characteristics c. Format of the subtitle track(s) d. Metadata tracks and presence of media track events e. Ingest protocols 2. Instructions such as: a. CMAF/DASH publishing profile b. Configuration of Switching sets: i. Ladder of bit rate/quality ii. The encoder configuration for each track iii. Nominal and maximum segment duration iv. Frequency and characteristics of random access and switching points v. Initialization segment characteristics vi. Content protection scheme and encryption mode vii. Metadata and annotation such as roles, languages, ratings, and accessibility viii. Metadata tracks associated with the switching set or individual tracks ix. Media track events to be included in each track x. Chunk encoding for low latency streaming c. Period structure, splicing opportunities, and conditions d. 5GMSd AS URL exposed at M4 e. Segments Addressing modes f. Trick mode and thumbnail navigation tracks g. Service description h. Content splicing/ad opportunity signaling i. Subtitle generation from the uplink ingest audio track(s) j. Multi-language support (when audio tracks for two or more languages are ingested). k. Manifest format, annotations, and metadata

### 5.2.4	Collaboration Scenarios

### 5.2.4.1	Content preparation before downlink streaming

In this collaboration, the 5GMSd Application Provider requests content preparation for its stream before distribution. Figure 5.2.4.1-1 shows such a scenario.

UE

5GMSd

Client

5GMSd

Aware Application

5GMSd AS

External DN

5GMSd

Application Provider

M1d

M2d

Trusted DN

M4d

5GMSd AF

M5d

PCF

M8d

N5

Figure 5.2.4.1-1: Content preparation before downlink streaming

In this case:

1.	The Application Provider defines the required content preparation and requests the 5GMSd AF to create the process using the Content Preparation Template (CPT) through M1d.

2.	The 5GMSd AF configures the 5GMSd AS according to the media transformation(s) specified in the Content Preparation Template (M3d) and responds to the Application Provider (M1d).

3.	The media is streamed from Application Provider to 5GMSd AS (M2d).

4.	The content is transformed by 5GMSd AS according to the Content Preparation Template and provided for distributions (M4d).

5.	The Application Provider may update, retrieve, or destroy the Content Preparation Template using M1d.

Editor’s Note: How does DistributionConfiguration relate to CPT? Does the CPT define the initial/generic formats (such as HLS and DASH) and DistributionConfiguration defines a subset of it for distribution?

### 5.2.4.2	Content preparation after uplink ingest streaming

In this collaboration, the 5GMSu Application Provider or 5GMSu Aware Application requests content preparation for the uplink ingest stream and delivery to 5GMSu Application Provider. Figure 5.2.4.2-1 shows such a scenario.

UE

5GMSu

Client

5GMSu

Aware Application

5GMSu AS

External DN

5GMSu

Application Provider

M1u

M2u

Trusted DN

M4u

5GMSu AF

M5u

PCF

M8u

N5

Figure 5.2.4.2-1: Content preparation after uplink ingest streaming

In this case:

1.	The Application Provider defines the required content preparation and requests the 5GMSu AF to create the process using the Content Preparation Template (CPT) through M1u.

2.	The 5GMSu AF configures the 5GMSu AS according to the media transformation(s) specified in the Content Preparation Template (M3u) and responds to the Application Provider (M1u).

3.	The media is streamed from 5GMSu Client to 5GMSu AS (M4u).

4.	The content is transformed by 5GMSu AS according to the Content Preparation Template and egested to the Application Provider (M2u).

5.	The Application Provider/Application may update, retrieve, or destroy the Content Preparation Template using M1u/M5u.

### 5.2.4.3	Content preparation between uplink ingest and downlink streaming

In this collaboration, the 5GMSu Application Provider requests content preparation for the uplink ingest stream from one UE and before downlink streaming to other UEs. Figure 5.2.4.3-1 shows such a scenario.

UE

5GMSu Client

5GMSu Aware Application

5GMSu AS

External DN

5GMS Application Provider

M1u

Trusted DN

M4u

5GMSu AF

M5u

PCF

M8u

N5

5GMSd AS

5GMSd AF

M1d

UE

5GMSd Client

5GMSd Aware Application

M4d

M5d

M3u

M3d

I2

I1

M8d

Figure 5.2.4.3-1: Content preparation after uplink ingest streaming

In this case:

1.	The Application Provider defines the required content preparation and requests the 5GMSd AF to create the process using the Content Preparation Template (CPT) through M1d.

2.	The 5GMSd AF configures the 5GMSd AS according to the media transformation(s) specified in the Content Preparation Template (M3d) and responds to the Application Provider (M1d).

3.	The 5GMSu AF configures the 5GMSu AS according to the media transformation(s) specified in the Content Preparation Template (M3u) and responds to the Application Provider (M1u).

4.	The content is transferred to 5GMSd AS and transformed according to the Content Preparation Template (I2). I2 is an implementation specific interface and is considered out of scope of this document.

5.	The content is streamed to the 5GSd Client from 5GMSd AS (M4d).

6.	The Application Provider/Application may update, retrieve, or destroy the Content Preparation Template (M1d).

NOTE 1: 	In the above steps, the I1 interface is not used for the simplicity. In section 5.6, two alternative call flows are presented that use I1 interface. I1 is an implementation specific interface and is considered out of scope of this document.

NOTE 2: 	In the deployment of this collaboration scenario, 5GMSu AS and 5GMSd AS may be co-located and therefore the I2 interface is not exposed. Similarly, 5GMSu AF and 5GMSd AF may be co-located.

### 5.2.5	Deployment Architectures

Editor’s Note: Based on the 5GMS Architecture, develop one or more deployment architectures that address the key topics and the collaboration models.

### 5.2.6	Mapping to 5G Media Streaming and High-Level Call Flows

#### 5.2.6.1	Call flow for content preparation before downlink streaming

The call flow is shown in Figure 5.2.6.1-1.

Figure 5.2.6.1-1: High-level call flow for content preparation before downlink streaming

Steps:

1.	The 5GMSd Application Provider creates a Provisioning Session with the 5GMSd AF (M1d).

2.	The 5GMSd Application Provider provisions the 5GMSd AF with one or more Content Preparation Templates defining instructions for content preparation, as well as the required output format(s) (M1d).

3.	The 5GMSd AF, based on the received Content Preparation Templates, requests the 5GMSd AS to verify availability of resources for content preparation (M3d, procedures not specified):

a.	Syntax checking of Content Preparation Template(s).

b.	Semantic validation of Content Preparation Template(s).

c.	Basic assessment of 5GMSd AS resource availability implied by the Content Preparation Template(s).

4.	The 5GMSd AF acknowledges to the 5GMSd Application Provider the successful creation of Content Preparation Templates (M1d).

5.	The 5GMSd Application Provider provisions the 5GMSD AF with a Content Hosting Configuration that references one or more Content Preparation Templates created in step 2 above (M1d).

6.	The 5GMSd AF, based on the received Content Hosting Configuration, requests the 5GMSd AS to confirm the availability of distribution resources (M3d, procedures not specified).

7.	The 5GMSd AF acknowledges to the 5GMSd Application Provider the successful creation of the Content Hosting Configuration (M1d).

8.	The 5GMSd Application Provider feeds the content to the 5GMSd AS (M2d).

9.	The 5GMSd Application Provider optionally provides the service access information to the 5GMSd-Aware Application (M8d, out of scope)

If needed, steps 10–14 may optionally be executed:

10.	The 5GMSd-Aware Application requests the 5GMSd Application Provider to use the service (M8d).

11.	The 5GMSd Application Provider provides the 5GMSd AF with updated Content Preparation Template(s) (M1d).

12.	The 5GMSd AF, based on the modified Content Preparation Template(s), requests an updated confirmation of downlink streaming resource availability (M3d).

13.	The 5GMSd AF acknowledges to the 5GMSd Application Provider that the Content Preparation Template(s) have been successfully updated (M1d).

14.	The 5GMSd Application Provider acknowledges to the 5GMSd AF the use of the service (M8d).

The rest of the call flow concerns the 5GMS downlink streaming process:

15. The 5GMS-Aware Application request the 5GMSd Client to start an uplink streaming session (M6d/M7d).

16.	If Service Access Information was not provided in step 9, the 5GMSd Client requests this information from the 5GSMd AF (M5d).

17.	 The 5GMSd Client requests start of the downlink streaming session from the 5GSMd AF (M5d).

18.	The 5GMSd AF requests instantiation of the content preparation process (M3d).

19.	The 5GMSd AS instantiates the media preparation process if it (or any of its parts) are not already running.

20.	The 5GMSd AF acknowledges the instantiation of the content preparation process (M3d).

Steps 18–20 are not needed if another user has already requested the content and therefore the content preparation process is already running on the 5GMS AS, or if this process was instantiated in earlier steps of the workflow (e.g. in step 3 or step 12).

One use-case for steps 18–20 is when a user requests a content format that is not included in the previously instantiated content preparation process. In this case, 5GMSd AS may add new processes to the content preparation processing to provide the requested content format.

21. The downlink media streaming starts (M4d).

Finally:

22.	The 5GMSd AS releases its resources after observing a period of interactivity.

NOTE:	Step 22 is implementation-dependent.

#### 5.2.6.2	Call flow for content preparation after uplink streaming

The call flow is shown in Figure 5.2.6.2-1.

Figure 5.2.6.2-1: Call flow for content preparation after uplink streaming

Steps:

1.	The 5GMSu Application Provider creates a Provisioning Session with the 5GMSu AF (M1u).

2.	The 5GMSu Application Provider creates one or more Content Preparation Templates defining instructions for content preparation, as well as the required output format(s) (M1u).

3.	The 5GMSu AF, based on the received Content Preparation Templates requests the 5GMSu AS to verify availability of content preparation and contribution resources, e.g. to allocate 5GMSu content ingest and contribution egest resources. (M3u, procedures not specified):

a.	Syntax checking of Content Preparation Template.

b.	Semantic validation of Content Preparation Template.

c.	Basic assessment of 5GMSu AS resource availability implied by the Content Preparation Template.

4.	The 5GMSu AF acknowledges to the 5GMSu Application Provider the successful creation of Content Preparation Templates, as well as successful provisioning of content preparation (M1u).

5.	The 5GMSu Application Provider optionally provides the service access information to the 5GMSu-Aware Application (M8u, out of scope)

If needed, steps 6–10 may optionally be executed.

6.	The 5GMSu-Aware Application requests the 5GMSu Application Provider to support impending reception of uplink streaming content (M8u)

7.	The 5GMSu Application Provider provides the 5GMSu AF with updated Content Preparation Template(s) (M1u).

8.	The 5GMSu AF, based on the modified Content Preparation Templates, requests an updated confirmation of uplink streaming resource availability (M3u).

9.	The 5GMSu AF acknowledges to the 5GMSu Application Provider that the Content Preparation Templates have been successfully updated (M1u).

10.	The 5GMSu Application Provider acknowledges to the 5GMSu AF the use of the service (M8u).

The rest of the call flow concerns the 5GMS uplink streaming process:

11.	The 5GMSu-Aware Application request the 5GMSu Client to start an uplink streaming session (M6/7u).

12.	If Service Access Information was not provided in Step 5, the 5GMSu Client requests this information from the 5GSMu AF (M5u).

13.	The 5GMSu Client requests start of the uplink streaming session from the 5GSMu AF (M5u).

NOTE 1:	Although the above step is defined by the stage 2 design in TS 26.501 [15], it is not realised in Release 16 by stage 3 procedures defined in TS 26.512 [16].

14.	The 5GMSd AF requests instantiation of the content preparation process (M3u).

15.	The 5GMSd AS instantiate the media preparation process if it has not started before (M3u).

16.	The 5GMSd AF acknowledges the instantiation of the content preparation process (M3u).

Steps 14–16 may not be needed if the content preparation process has been instantiated during earlier steps in the call flow (such as between 3 and 8).

17.	Uplink media streaming from the 5GMSu Client to the 5GMSu AS commences (M4u).

18.	Media streaming egest from the 5GMSu AS to the 5GMSu Application Provider (M2u) commences.

Finally:

19.	The 5GMSu AS releases its resources after observing a period of interactivity. Note that this is implementation dependent.

NOTE 2:	The 5GMSu Application Provider also sets up the M2u configuration during the provisioning section. The details of such set-up are not shown in the above call flow.

Editor’s Note: The M2u configuration process is the subject of the uplink streaming topic of this study.

#### 5.2.6.3	Baseline call flow for content processing between uplink streaming and downlink streaming

Figure 5.2.6.3-1 shows the call flow for this scenario.

Figure 5.2.6.3-1: High-level call flow for content preparation
between uplink streaming and downlink streaming

Steps:

1.	Identical to steps 1–7 in 5.2.6.1-1.

2.	The 5GMS Application Provider creates a Provisioning Session with the 5GMSu AF (M1u).

3.	The 5GMSu AF requests the 5GMSu AS to confirm the uplink resources availability. (M3u, procedures not specified).

4.	The 5GMSu AF acknowledges to the 5GMSu Application Provider of the successful provisioning (M1u).

5.	The 5GMS Application optionally provides the service access information to the 5GMSu-Aware Application (M8u, out of scope).

6. The 5GMS-Aware Application request the 5GMSu Client to start an uplink streaming session (M6/7u).

7.	If Service Access Information was not provided in step 5, the 5GMSu Client requests this information from the 5GSMu AF (M5u).

8.	 The 5GMSu Client requests start of the uplink streaming session from the 5GSMu AF (M5u).

9. The uplink media streaming starts (M4u).

10.	The 5GMSu AS streams the content to the 5GMSd AS (I2, not specified).

11.	The 5GMS Application Provider optionally provides the service access information to the 5GMSd-Aware Application (M8d, out of scope).

12.	Identical to steps 10–14 in 5.2.6.1-1.

13.	Identical to steps 15–21 in 5.2.6.1-1.

14.	The 5GMSu AS releases its resources after observing a period of interactivity.

NOTE 1:	Step 14 is implementation-dependent.

15.	The 5GMSd AS releases its resources after observing a period of interactivity.

NOTE 2:	Step 15 is implementation-dependent.

### 5.2.7	Potential open issues

#### 5.2.7.1	Open issues in collaboration scenario 1: Content preparation before downlink streaming

Open issue 1: Content Preparation Template information

TS 26.512 [16] only mentions that the format of the Content Preparation Template is identified by its MIME type. At least one format needs to be defined for the instruction in Content Preparation Template for common services such as multi-rate streaming.

Open issue 2: Support of other protocols

The current specification only supports HTTP pull and DASH-IF ingest. Other protocols may need to be added.

Open issue 3: Address translation for complex pull requests

Figure 5.2.7.2-1 shows an example in which the content preparation takes segments n and n+1 from the upstream 5GMSd Application Provider and combines them into one segment with double duration for delivery to the 5GMSd Client. The goal could be to achieve higher compression efficiency for distribution.

5GMSd AS

Content Hosting

M2d

Content Preparation

Cache

M4d

5GMSd Client

5GMSd Application Provider

Request for Segment 246

Segment 246

Segment 247

Request for Segment 247

Segment 123

Request for Segment 123

Figure 5.2.7.2-1: An example of a media distribution by pull

Steps:

1.	The 5GMSd Client requests a media segment http://cdn.com/segment123.mp4 through M4d.

2.	The requested URL is mapped in the Content Hosting distribution configuration  (“cdn.com” => “originprovider.com/video/” and transformation of request URL using the rule {n} => {n, n+1}). Two separate requests are made via M2d to the 5GMSd Application Provider for the following media segments:

-	http://originprovider.com/video/segment246.mp4

-	http://originprovider.com/video/segment247.mp4

3.	The 5GMSd Application Provider provides the two requested media segments to the Content Preparation subfunction, which in turn merges the two segments into one, delivers the output segment to the Cache.

4.	The Cache subfunction delivers the merged media segment to the UE via an M4d response.

NOTE:	It is assumed that the initialization segment is available for initialization of the decoder in the content preparation module.

Table 7.6.3.1 1 in TS 26.512 [16] defines the content hosting configuration resource. Each Content Hosting Configuration may include one or more Distribution Configurations, each one with an optional Content Preparation Template. However, the Content Hosting Configuration is currently allowed to include only one Ingest Configuration.

The Ingest Configuration defines the input format for content preparation. Currently, two ingest protocols are specified in clause 8.2 of [16]: HTTP pull-based ingest and DASH-IF push-based. In the case of HTTP pull, if a request is received in M4d that can’t be satisfied, an HTTP pull request is made through M2. The PathRewriteRules currently defined in clause 7.6.3 of [16] do not address step 2 of the above example (i.e. transforming one request into two different requests).

#### 5.2.7.2	Open issues in collaboration scenario 2: content preparation after uplink streaming

In addition of the above issues of collaboration scenario 1, the following issues may be considered.

In this case, the content is delivered to the 5GMSu Application Provider through M2u.

Open issue 4: Egest protocols

TS 26.512 [16] Table 8.1-1 defines the ingest protocols at M2d. But the egest protocols for M2u are not defined. One possibility is use already defined ingest protocols also as possible egest protocols. However, it must be investigated whether the current definitions of these protocols are adequate for egest.

Editor’s Note: We recommend addressing this issue in uplink streaming topic of this study.

Open issue 5: Egest Configuration

TS 26.512 [16] Content Hosting Configuration resource defines an Ingest Configuration. It is not clear whether the same resource can be used for Egest configuration and whether the parameters are adequate.

Editor’s Note: We recommend addressing this issue in uplink streaming topic of this study.

#### 5.2.7.3	Open issues in collaboration scenario 3: content preparation between uplink and downlink

Since both uplink and downlink is used, the following issues should be considered.

Open issue 6: Signalling the connection between the uplink and downlink

Assuming the previous issues are addressed, then the Content Hosting Configuration resource for uplink and downlink would be separately used in M1u and M1d. The uplink Content Hosting Configuration’s egest configuration should be aligned with the downlink Content Hosting Configuration’s ingest configuration as is shown in the following figure.

The following issues must be addressed:

1.	Is the protocol left to the network operator and only direct connection between two Application Servers signalled?

2.	Is any (optional) standard protocol needed for the connection? If so, which protocols?

3.	Are the current Ingest Configuration’s (and its counterpart for Egest) parameters adequate for setting up the connection between uplink and downlink?

4.	Do the connection configuration and 5GMS Application Server’s addresses need to be hidden from the 5GMS Application Provider?

### 5.2.8	Candidate Solutions

#### 5.2.8.1	Content Preparation Template requirements

##### 5.2.8.1.1	Unencrypted single CMAF track to single unencrypted CMAF switching set

The Content Preparation Template must define the following parameters:

1.	The address/location of the input CMAF segments.

2.	Output CMAF switching set configuration:

a.	Output manifest parameters: The characteristics that are typically described in a manifest such as an MPEG-DASH MPD [11]. While these parameters are per track, they can be described once if one or more of them are common across tracks. Examples:

i.	Packaging parameters: Container profile, codec/profile/level, bit rate, container profiles, maximum SAP period, start with SAP.

ii.	Video parameters: Width, height, sample aspect ratio, frame rate.

iii.	Audio parameters: sampling rate, audio channel configuration.

b.	Internal encoding parameters: The parameters used for encoding each track that are not presented in the output manifest, such as bit rate control, motion search area, and algorithm, CBR/VBR/Capped VBR encoding, use of specific quality metrics.

i.	Common encoding parameters (usually common in a codec/profile/level).

ii.	Vendor-defined (implementation-specific) parameters.

#### 5.2.8.2	Content Preparation Template candidates

##### 5.2.8.2.1	CMAF input format candidate 1: DASH MPD manifest

In this case, the characteristics of the input CMAF track can be described by a DASH MPD manifest in order to correctly initialise the media transcoder in the Content Preparation subfunction of the 5GMS AS. The manifest may include information such as codec/profile/level, as well as general characteristics of the media, such as maximum segment duration, video width, height and frame rate, the existence of any SEI messages and other metadata, the existence of any events schemes, as well as the location of each segment in the input CMAF track. In the case of uplink media streaming, where HTTP pull is used to egest the transcoded segments via M2u, the manifest can even optionally define the locations of CMAF segments on the 5GMSu AS.

##### 5.2.8.2.2	CMAF input format candidate 2: A new document format

A new document format can be used for describing the input CMAF segments. The advantage of such a solution is when the same format is used for describing the output CMAF formats.

##### 5.2.8.2.3	CMAF output format candidate 3: Extended manifest format

In this approach, a standard manifest format is used for describing the output manifest parameters, but it is extended to also carry the internal encoding parameters (both common and vendor-defined).

An example is to use MPEG-DASH MPD format and add descriptors to the adaptation set and/or representations for the internal encoding parameters. Two classes of descriptors can be added:

1.	The common encoding descriptor per codec, carrying common parameters.

2.	Vendor-specific descriptors, carrying vendor-defined parameters.

Since the MPD essential and supplemental descriptor syntax allows different scheme URIs to be defined, both of the above features can be expressed using the same descriptor data type.

##### 5.2.8.2.4	CMAF output format candidate 4: Manifest with supplementary encoding parameters document

In this approach, a standard manifest format is used to describe the output manifest parameters and a separate document is used to describe the internal encoding parameters.

An example is the following elements:

1.	MPEG-DASH MPD format for output manifest parameters.

2.	A JSON document containing an array of objects that each include a DASH Representation identifier referencing a Representation in the MPD. Each object also includes encoding parameters for the Representation. The common encoding parameters may be extended with vendor-specific parameters inside a child object tagged with a URI that uniquely identifies the vendor.

For example, the MPD in Listing 5.2.8.2.4-1 below has two representations: R1 and R2. (For simplicity the adaptation set is not shown.) The internal encoding parameters document in Listing 5.2.8.4.-2 has a JSON array containing two elements. Each element refers to one DASH Representation in the MPD by its id attribute value. Each array element includes the encoding parameters for the corresponding Representation. The second element includes a vendor-specific encoding parameter that is signaled using the vendor URN. The other items in that object are defined by the vendor.

Listing 5.2.8.2.4-1: MPEG-DASH MPD

<MPD> <Representation @id='R1' …> <Representation @id='R2' …> </MPD>

Listing 5.2.8.2.4-2: Supplementary encoding parameters document in JSON format

[{ Rid = 'R1'; search-window =64; frame-rate= 30; …}; { Rid = 'R2'; search-window =64; frame-rate= 60; {vendor='urn:companyA:encoding:CAE', mode='segment-based', context='sports' …}];

##### 5.2.8.2.5	CMAF output format candidate 5: A document defining both the output manifest and encoding parameters

In this approach, a new document format is defined to describe both the manifest output parameters and the internal encoding parameters.

An example of such a solution would be a JSON document containing an array of objects  that each include the following information:

1.	Output manifest parameters.

2.	Common internal encoding parameters.

3.	Vendor-specific internal encoding parameters tagged with the vendor’s identifier (such as a URI).

Another alternative would be to use the DASH Industry Forum’s Content Protection Information Exchange Format (CPIX) [38] and possibly extend it to carry additional parameters that are needed.

#### 5.2.8.3	Combining the Content Preparation Template candidate solutions

Since both input, outputs and encoding information need to be provided in the Content Preparation Template, the following solutions are possible for the overall template by combining the candidate solutions described in clause 5.2.8.2 above:

1.	Single MPD:

a.	One adaptation set with one input representation describing the input according to 5.2.8.2.1

b.	One adaptation set with multiple input representation describing the output tracks according to 5.2.8.2.3

2.	A document consisting of two MPDs, with possibly a supplementary document:

a.	One MPD describing the input according to 5.2.8.2.1 and

b.	One of the following:

i.	One MPD describing the outputs and encoding format according to 5.2.8.2.3, or

ii.	One MPD describing the outputs and one document describing the encoding parameters according to 5.2.8.2.4.

3.	Single JSON document:

a.	One item describing the input representation according to 5.2.8.2.2, and

b.	An array of objects according to 5.2.8.2.5, each of which describes:

i.	One output.

ii.	The encoding parameter for that output.

#### 5.2.8.4	Combined CMAF input and output formats candidate: NBMP Workflow Description Document

The NBMP Workflow Description Document (WDD) [39] can describe the entire workflow. In this use case, the WDD describes the input format, as well as the array of tasks/function instances, each of which defines the CMAF output track as well as the encoding parameters for that track, as is shown in the following figure:

5GMSd AS

Content Preparation

Task 1:

Transcoder 1

Task 2:

Transcoder 2

Task n-1:

Transcoder n-1

M2d

Content Hosting

NBMP WDD and/or function template

Figure 5.2.8-1: Using NBMP WDD to describe CMAF content preparation

The NBMP WDD in this case describes the input CMAF as the input of workflow, and the function, configurations, and output of each task. Since many features of the NBMP specification are not used in this specific workflow, the WDD features can be profiled to a suitable subset of descriptors defined by NBMP in [39].

The NBMP specification allows so-called function templates to be defined. One way to simplify the support for NBMP by 5GMSd AS would therefore be to define a function template for each Content Preparation Use Case. For example, the 5GMS CMAF Content Preparation function template could define (among other things):

1.	Input CMAF media profile using explicit description: MPEG-DASH MPD or HLS m3u8 playlist.

2.	The push/pull protocols for ingesting CMAF content at M2d.

3.	The required CMAF output formats.

4.	The transcoder’s common and vendor-specific configuration parameters

5.	Multiple codec output.

6.	Reporting, monitoring and notification parameters for each transcoding function.

Another advantage of the NBMP WDD format is that it can be used to describe other Content Preparation use cases and therefore one single format may be able to address several applications.

In this approach:

1.	A function reference template is created for the CMAF content preparation

2.	A profile of NBMP spec is defined to simplify the NBMP Workflow requirements.

#### 5.2.8.5	Address translation for complex pull requests

Open issue 3 of 5.2.7.1 provide a gap in address translation. There are three possibilities for addressing this gap:

1.	Keep PathRewriteRules as is, and therefore exclude content preparation cases with more than one input and/or with more complex translation to be used with push protocol(s) only.

2.	Extend the current request URL transformation capabilities of the 5GMSd Content Hosting feature.

3.	Keep PathRewriteRules as is and include an additional (and more sophisticated) address translation function as a part of the Content Preparation subfunction of the 5GMSd AS.

Figure 5.2.8.5-1 shows the architecture for the third option above.

5GMSd AS

Content Hosting

M2d

Content Preparation

PathRewriteRules

Cache

M4d

5GMSd Client

5GMSd Application Provider

Request for Segment 246

Segment 246

Segment 247

Request for Segment 247

Segment 123

Request for Segment 123

Address Translator

Figure 5.2.8.5-1: A candidate solution for the example of media distribution by pull

Steps:

1. The 5GMSd Client requests a media segment http://cdn.com/segment123.mp4 through M4d.

2. The requested URL is mapped according to the PathRewriteRules in the Content Hosting distribution configuration and provided to the address translator of the Content Preparation module.

3. The address translator module transforms the received request into two separate requests via M2d to the 5GMSd Application Provider for the following media segments:

-	http://originprovider.com/video/segment246.mp4

-	http://originprovider.com/video/segment247.mp4

4.	The 5GMSd Application Provider provides the two requested media segments to the Content Preparation subfunction, which in turn merges the two segments into one, delivers the output segment to the Cache.

5.	The Cache subfunction delivers the merged media segment to the UE via an M4d response.

NOTE:	It is assumed that the initialization segment is available for initialization of the decoder in the content preparation module.

### 5.2.9	Conclusion and recommendations

This study explored three deployment scenarios for content preparation:

1)	content preparation before downlink distribution,

2)	content preparation after uplink streaming, and

3)	content preparation between uplink streaming and downlink distribution.

The call flows for all deployment scenarios are defined along with the gap analysis of TS 26.512 in addressing those scenarios.

As this study shows, the use of the content preparation template is not explained adequately in TS 26.501. Furthermore, TS 26.512 needs several extensions to make the use content preparation template interoperable in 5GMSA.

The following extensions are recommended:

-	Inclusion of content preparation deployment scenarios and associated call flows

## 5.3	Traffic Identification

### 5.3.1	Description

For different features within the 5G Media Streaming Architecture, it is necessary for the 5G System to identify the traffic flows. The increased usage of transport encryption (e.g. HTTPS) increases the difficulty of detecting the packets for certain application flows. Existing detection methods, such as using “significant parts of the URL to be matched” (contained in a Packet Flow Description, see below), are impractical for HTTPS traffic, since the URL is carried in the encrypted payload.

Multimedia streaming applications might not be able to uniquely identify the 5-tuple of the streaming session, since the 5-tuples are often changing. This is due to factors such as load balancing, CDN distribution, multiple concurrent requests for different types of resources, etc. This study will address how to properly configure the 5G System to enable efficient detection of application flows (service data flows) e.g. for event reporting, and QoS profile usage, etc.

Note that the TS 23.50x specifications use different terminology from the TS 29.xxx specifications. Furthermore, TS 23.503 [41] uses slightly different terms than TS 23.501 [23] and TS 23.502 [24]. The two common terms are defined in TS 23.503:

-	Packet flow: A specific user data flow from and/or to the UE.

-	Service data flow: An aggregate set of packet flows carried through the UPF that matches a service data flow template.

The terms traffic detection [23] and application detection [23] refer to the process of finding matching service data flows among all packet flows. This logic is defined in TS 23.503 as an application detection filter.

The procedures in TS 23.502 use the term flow description, which is only a subset of an IP Packet Filter Set (as defined in TS 23.501).

Figure 5.3.1-1 depicts the chain of functions (taken from TS 29.244 [26], Figure 5.2.1-1) within an UPF.

Figure 5.3.1-1: Packet processing flow in the UP function (Figure 5.2.1-1 from TS 29.244 [26])

The steps are as follows:

1.	The UPF always first looks up the Packet Forwarding Control Protocol (PFCP) session context to which a packet belongs. The PFCP session context is an individual PDU session or a standalone PFCP session not tied to any PDU session.

2.	Then there are so-called Packet Detection Rules (PDR), which implement traffic detection of the service data flows with respect to different conditions.

NOTE:	A PDR is direction specific. Thus, an Uplink (UL) PDR and a Downlink (DL) PDR are needed to detect a bidirectional Service Data Flow.

3.	Based on the PDR result, the next rules are executed, namely Multi-Access Rule (MAR), Forward Action Rule (FAR), QoS Enforcement Rule (QER), and Usage Reporting Rule (URR).

NOTE:	Only the Forward Action Rule (FAR) is mandatory. The QoS Enforcement Rule (QER) is only present for QoS Flows. The Usage Reporting Rule (URR) is only available when traffic volume measurements (e.g. for charging) are needed.

The Packet Detection Rule (PDR) is based on Service Data Flow Templates, which contain one or more Service Data Flow (SDF) Filters or an Application Identifier. An Application Identifier refers to one or more Packet Flow Descriptions (PFDs).

A Service Data Flow (SDF) Filter contains for IP PDU Sessions a single IP Packet filter, i.e. any combination of

-	Source/destination IP address or IPv6 prefix.

-	Source / destination port number.

-	Protocol ID of the protocol above IP/Next header type.

-	Type of Service (ToS) (IPv4) or Traffic class (IPv6) and Mask.

-	Flow Label (IPv6).

-	Security parameter index.

-	Packet Filter direction.

A PFD includes a PFD ID; and one or more of the following:

-	3-tuple(s) comprising protocol, server-side IP address and port number.

-	the significant parts of the URL to be matched, e.g. host name.

-	a domain name matching criterion and information about applicable protocol(s).

The application detection filter can be configured in the SMF and the SMF then provides it in the Service Data Flow Template to the UPF. Alternatively, the Service Data Flow Template for traffic handling in the UPF is received from the dynamic PCC Rule.

Besides, the Management of Packet Flow Descriptions enables the UPF to perform accurate application detection when PFD(s) are provided by an Application Service Provider (ASP) and then to apply enforcement actions as instructed in a PCC Rule.

The operator is able to configure pre-defined PCC Rules in the SMF or dynamic PCC Rules in the PCF. A PCC rule includes either a list of Service Data Flow filters or an application identifier for Service Data Flow detection.  The PCC rule further includes charging control information, i.e. charging key and optionally a Sponsor identifier or an ASP identifier or both.

The application identifier references one or more PFDs, which are managed using the PFD Management API. Depending on the service level agreements between the operator and the Application Server Provider, it may be possible for the ASP to provide to the SMF individual PFDs or the full set of PFDs for each application identifier maintained by the ASP via the PFD Management service in the NEF (PFDF). The PFDs become part of the application detection filters in the SMF/UPF and are thereafter used as part of the logic to detect traffic generated by an application. The ASP may remove or modify some or all of the PFDs which have been provided previously for one or more application identifiers. The SMF may report the application stop to the PCF for an application instance identifier as defined in clause 5.8.2.8.4 of TS 23.501 [5] if the removed/modified PFD in SMF/UPF would result in an inability to detect traffic for that application instance.

The ASP manages (i.e. provisions, updates, deletes) the PFDs through the NEF (PFDF). The PFD(s) are transferred to the SMF through the NEF (PFDF). The PFDF is a logical functionality in the NEF which receives PFD(s) from the ASP through the NEF, stores the PFD(s) in the UDR and provides the PFD(s) to the SMF(s) either on the request from ASP PFD management through NEF (PFDF) (push mode) or on the request from SMF (pull mode). Finally, the PFDF functionality is a service provided by the NEF.

The ASP may provide/update/remove PFDs with an allowed delay to the NEF (PFDF). Upon reception of the request from the ASP, the NEF (PFDF) checks if the ASP is authorized to provide/update/remove those PFD(s) and request the allowed delay. The NEF (PFDF) may be configured with a minimum allowed delay based on SLA to authorize the allowed delay provided by the ASP. When both the requesting ASP and the requested allowed delay are successfully authorized, the NEF (PFDF) translates each external Application Identifier to the corresponding Application Identifier known in the core network. The NEF (PFDF) stores the PDF(s) into the UDR.

The Application Identifier is simply an index to a set of application detection rules configured in the UPF. It is an identifier that can be mapped to a specific application traffic detection rule.

The procedure is depicted Figure 5.3.1-2 below:

Figure 5.3.1-2:

The PFD (Packet Flow Description) is a set of information enabling the detection of application traffic.

Each PFD may be identified by a PFD ID. A PFD ID is unique in the scope of a particular Application Identifier. Conditions for when a PFD ID is included in the PFD are described in TS 29.551 [6]. There may be different PFD types associated with an Application Identifier.

### 5.3.2	Collaboration Scenarios

#### 5.3.2.1	General Collaboration Scenarios

The 5GMSd Application Provider negotiates with the MNO an SLA to provide differentiated treatment, including network QoS and charging for its 5GMSd-Aware Application. The Application Provider provides the necessary information to the MNO to detect the traffic, to ensure its correct and exclusive identification. The MNO detects the traffic correctly and applies the agreed traffic treatment.

Due to privacy concerns, the content hosting is provided by the Application Provider in an external Data Network. However, the 5GMSd Application Provider leverages the network features either via a 5GMSd AF in the external Data Network (Figure 5.9.2-1) or via a 5GMSd AF in the trusted Data Network (Figure 5.9.2-2).

Figure 5.9.2-1: Collaboration 1 (Collaboration 3 of TS 26.501)

Figure 5.9.2-2: Collaboration 2 (Collaboration 4 of TS 26.501)

In order to use flow-based network features (such as different QoS classes or different charging policies), the 5G System needs to detect the relevant traffic.

#### 5.3.2.2	Differentiated Services/ToS-enabled Collaboration Scenarios

Differentiated Services (DS) [76, 77] is a scalable scheme for managing application traffic by classifying the traffic into a set of coarse-grained traffic classes. A Differentiated Service (DS) domain is a continuous set of DS-capable routers, which are operated with a common set of configurations. Each IP packet in a DS domain is marked and conditioned according to its traffic class. A 6-bit DS Code Point (DSCP) of the 8-bit differentiated services field (DS field) is used for marking. The DS field replaces the ToS field in the IPv4 packet headers and the Traffic Class field in the IPv6 header.

End host systems may mark IP packets with a specific DSCP value prior to transmission. DS-enabled routers treat the packet according to the DSCP value when performing routing operations on it. Border gateway routers typically mark packets with a DSCP value based on some traffic policy, overriding any value set by hosts.

The RFCs defining Differentiated Services recommend a set of Per-Hop Behaviors (PHB), namely:

-	Default Forwarding (DF) PHB, defined in section 4.1 of RFC 2474 [76], is used for traffic without special treatment.

-	Class Selector PHB, defined in section 4.2.2.2 of RFC 2474 [76] is used for maintaining backwards compatibility with the IP precedence field of ToS.

-	Expedited Forwarding (EF) PHB, defined by RFC 3246 [78], is dedicated to low loss or low latency traffic.

-	Assured Forwarding (AF) PHB, defined by RFC 2597 [79], offers different levels of forwarding assurances.

The DS domain operator can also implement additional custom PHBs.

In the context of ToS-based traffic identification and separation, it is reasonable to assume the Data Network north of the UPF (N6) is DS-enabled. The 5G System is embedded in a larger DS domain, using same ToS values across multiple devices in order to provide Quality of Service Support like a DSCP-enabled link. However, it is not required to deploy DS capable routers for using in order to use the ToS field in the IP packet filter set for traffic identification.

According to clause 4.1 of TS 26.501 [15], the 5GMS functions may be deployed within the trusted Data Network or an external Data network. As noted above, DS Code Points are often reset at network domain borders, but not always. There may be deployments e.g., with localized Edge Computing or with direct peering realizations, where the DSCP values can be used up to the 5GMSd AF and/or 5GMS AS in an external Data Network. In this case, the logical DS domain is extended to include those externally-deployed 5GMS functions.

Figure 5.3.2.2-1 illustrates a deployment with a DS domain between the 5G System and the 5GMS functions deployed in the external DN. (The model is also valid for deployments in which the 5GMS functions both reside in the trusted DN.)

Figure 5.3.2.2-1: 5GMS deployment within a DS domain

Figure 5.3.2.2-2 illustrates a deployment with a DS domain between the 5G System and an externally deployed 5GMSd AS. The 5GMSd AF is deployed in the trusted DN.

Figure 5.3.2.2-1: 5GMS deployment within a DS domain

### 5.3.3	Deployment Architectures

The following figure depicts a potential architecture design for the realization of traffic detection. The architecture shows the involved network functions in the traffic detection.

Figure 5.3.3-1: Relevant architecture components

### 5.3.4	Mapping to 5G Media Streaming and High-Level Call Flows

#### 5.3.4.1	General

The Service Data Flow Templates support multiple different combinations to define parameters for traffic detection. This clause describes the common parameter combinations to detect specifically media streaming application traffic.

The Service Data Flow Template can take the form of either Service Data Flow filters (i.e. IP Packet Filter Sets) or an Application Id referencing Packet Flow Descriptions (PFDs).

An IP Packet Filter Set can contain different combinations of parameter values. Unspecified parameter values in the IP Packet Filter Sets are used to match any value of the corresponding information in the header of an IP packet. Common IP Packet Filter Set combinations are:

-	5-Tuple: The source and destination IP addresses, source and destination port numbers (potentially expressed as a small range of values) and the Protocol ID. This method of traffic detection is further described in clause 5.3.4.2 below.

-	ToS: The source IP address and the Type of Service (ToS). This method of traffic detection is further described in clause 5.3.4.3 below.

NOTE:	The Type of Service field is used here to map an application data flow to a specific PCC rule.

A Packet Flow Description (PFD) can contain different parameters. Common parameters are:

-	Domain Name: The Internet domain name of an application server. This method of traffic detection is not described further in the present document.

#### 5.3.4.2	Usage of 5-tuples for Traffic Identification

The application detection filters in the UPF can be configured based on a pre-configured PCC rule (i.e. in the SMF and provided to the UPF) or a dynamic PCC rule (i.e. provided by the PCF) By interacting with the PCF (possibly via the NEF) the 5GMS AF is able to provision, update and remove a dynamic PCC rule which contains Service Data Flow description parameters for traffic handling and traffic detection in the UPF.

When using 5-tuples for traffic detection, the following fields of the IP Packet Filter Set are used:

-	Source/destination IP address or IPv6 prefix.

-	Source/destination port number.

-	Protocol identifier of the protocol above IP/Next header type.

-	Packet Filter direction (uplink or downlink).

NOTE:	These fields are encoded in the Flow Description field, defined in clause 5.3.8 of TS 29.514 [28].

As shown in figure 5.3.4.2-1 (below), the 5GMSd AF in the external DN can send a request using Nnef_AFsessionWithQos API to provision, update or remove a request to reserve resources for a specific application/flow with specific flow descriptions. After the AF request authorization, the NEF interacts with the PCF, providing the flow description together with the QoS reference, the optional other parameters like Alternative Service Requirements, period of time or traffic volume, etc.

Figure 5.3.4.2-1:Flow description usage for traffic flow identification

If the request is authorised, the PCF determines the required QoS parameters based on the information provided by NEF/AF. After the Nnef_AFsessionWithQoS_Create Procedure, a transaction identifier is allocated by the NEF to identify this AF Session. Then the 5GMSd AF can subsequently invoke the Nnef_AFsessionWithQoS_Update API with this transaction identifier to update the flow description.

Alternatively, the 5GMSd AF in the trusted DN can directly send a request using Npcf_PolicyAuthorization API to provision, update and remove a request to reserve resources for a specific application/flow with specific flow descriptions.

Then the PCF initiates the PDU Session modification procedure to provide the updated PCC rule to the SMF and the SMF updates the PDRs in the UPF for the application/traffic identification and policy handling.

However, when a new TCP connection is opened and the old one is closed, the 5-tuple in the Flow Description needs to be changed. (This may be from the consequence of factors such as load balancing, multiple concurrent requests for different types of resources, use of a shared TCP connection pool, etc.) In such cases, the 5GMSd AF can invoke the NEF/PCF-related APIs with new flow description to update the PDRs installed in UPF to follow the changed transport layer 5-tuples for application/flow identification.

#### 5.3.4.3	Usage of ToS Traffic Class for Traffic Identification

The following is a simplified call flow when using the ToS Traffic Class for Traffic Identification, meaning, only the Type of Service field is used within a SDF Filter. The Type of Service (ToS) is an 8-bit field within the IP header (both IPv4 and IPv6) that can be used to convey a 6-bit DS Code Point (DSCP) value as described in clause 5.3.2.2, and for ECN marking [30]. Here, and in the candidate solution presented in clause 5.3.6, only the leading 6 bits are considered for traffic identification.

It is assumed here that the QoS flow should be used (e.g. for Premium QoS) as described in TS 26.512, Annex A.

Figure 5.3.4.3-1: ToS usage within an application traffic detection rule (simplified)

Figure 5.3.4.3-1 depicts a call flow for ToS-based traffic detection. It is assumed here that the 5GMSd AF provides the ToS value for traffic identification in the Policy Activation response message (step 5). Another solution might be that the Media Session Handler allocates a ToS value and then provides the value to the 5GMSd AF.

The call flow works as the following steps:

1:	The Media Session Handler activates a Dynamic Policy and provides the Policy Template Id with the activation request (among other parameters).

The 5GMSd AF triggers the activation of a Dynamic PCC rule:

2:	The 5GMSd AF uses the Policy Authorization Service API and triggers a PCC rule activation. The 5GMSd AF provides the UE IP address, an IP Packet Filter Set with the ToS value and the UE IP address of the requesting UE and QoS parameters.

3:	As result, the PCF uses the Npcf_SMPolicyControl APIs to provide a new PCC rule to the SMF.

4: 	The SMF uses the PDU Session Modification procedure to add/modify a QoS Rule in the UE SDAP entity.

5:	The SMF uses the N4 interface to provide a new Packet Detection Rule (PDR) together with other rules for the UE to the UPF. Once the new rule is installed in the UPF, the UPF starts taking actions on the detected traffic.

6:	If the Dynamic Policy can be activated, the 5GMSd AF provides a value for the ToS field in return.

NOTE 1: The ToS Value is not immediately provided to the Media Session Handler to prevent race conditions.

7:	The Media Player prepares a new TCP connection and sets the ToS value nominated by the 5GMSd AF on the TCP socket using the setsockopt() API or equivalent. As a result, all TCP packets for the flow will be marked by the UE with the ToS value.

8:	The TCP Connection is established, and the traffic is marked with the ToS field. The UPF detects the traffic (by inspecting the IP header) and handles it according to the policy in the PCC Rule.

NOTE 2:	The PCC Rule is scoped by the PDU Session, so the treatment of the ToS field value by the UPF is limited to the requesting UE. The UPF first looks up the relevant PDRs for a PDU session based on the incoming GTP Tunnel Id.

The UPF also needs to detect the downlink traffic matching the uplink traffic. There are different solutions to achieve this:

A:	The 5GMSd AS uses the same ToS field for downlink traffic as used for uplink traffic.

NOTE 3:	This solution may not work for cases where traffic crosses operational domain boundaries, since the ToS header field is often reset by border IP routers.

B:	The UPF captures the 5-tuple carrying a specific ToS field from the TCP SYN Packet that establishes the connection in the uplink direction. As result, the UPF automatically creates a new PDR in the opposite direction derived by inverting the address fields found in the SYN packet.

NOTE 4:	The connection handshake of other transport protocols may be more difficult to detect.

C:	Often, the UEs in a PLMN are shielded from public Internet traffic by means of firewalls that employ Network Address Translation (NAT). In order to set the ToS field within the Trusted DN to an appropriate value, the N6-NAT may set the downlink ToS to the same value as the uplink ToS.

NOTE 5:	This is similar to solution A above.

#### 5.3.4.4	Usage of Packet Flow Descriptions for Traffic Identification

The following are simplified call flows for the realization of the traffic identification leveraging Packet Flow Descriptions (PFDs).

In the first call flow (Figure 5.3.4.4-1) the provisioning step is described, in which one or more PFDs for the traffic identification of a single application are provisioned. The provisioned PFDs for the application are identified by the Application Identifier.

Figure 5.3.4.4-1: PFD Provisioning using the PFD Management API (simplified)

In the second call flow (Figure 5.3.4.4-2) the on-demand usage of a set of PFDs for an application is described.

Figure 5.3.4.4-2: PFD usage within an application detection filter (simplified)

Here, the 5GMSd AF resides within the Trusted Data Network and interacts directly with the PCF. When activating an QoS flow, the 5GMSd AF provides to the PCF the Application Identifer for detecting the 5G Media Streaming traffic together with the UE IP address and other policy information.

When the 5GMSd AF resides in an external Data Network, the 5GMSd AF instead uses the equivalent NEF API (such as the Nnef_AFsessionWithQoS or Nnef_ChargeableParty service APIs) and provides the (external) Application Identifier, referencing one or more PFDs, to the NEF.

The 5GMSd AF can update the PFDs during an ongoing media streaming session. When updating, the 5GMS AF uses the NEF PFD management APIs. The system then automatically updates the Packet Detection Rules in the UPF used for traffic detection.

### 5.3.5	Potential open issues

The exact behaviour and information that needs to be provided to and by the 5GMSd AF as well as the Media Session Handler need to be specified.

The following open issues have been identified:

1.	The Npcf_PolicyAuthorization API as defined in TS 23.502 [24] only supports usage of a flow description or an application identifier. The flow description is not further defined in TS 23.501 or TS 23.502. In Stage 3 specifications, a flow description represents only a 5-tuple. Other information elements of the Service Data Flow Filter are not supported.

2.	The Nnef_ChargeableParty and Nnef_AFsessionWithQOS APIs support usage of a flow description or an Application Identifier for referencing one or more PFDs. The flow description is not further defined in TS 23.501 or TS 23.502. Other information elements of the Service Data Flow Filter are not supported.

3.	The Npcf_PolicyAuthorization API Stage 3 as defined in TS 29.514 [42], only supports a flow description and a ToS value. However, it is not possible to define whether the ToS value should be used in uplink traffic detection or downlink traffic detection.

4.	The Nnef_AFsessionWithQOS and Nnef_ChargeableParty stage 3 APIs, as defined in TS 29.522 [43], only supports a Packet Flow Description (through the FlowInfo Type) or an Application Identifier for referencing one or more PFDs. Other information elements of the Service Data Flow Filter (like a ToS value) are not supported. Note, the FlowInfo type from TS 29.122 [44] is different from the FlowInformation type in TS 29.512 [45].

### 5.3.6	Candidate Solutions

#### 5.3.6.1	Overview

This section gives an overview of the different candidate solutions for application traffic flow identification within a PDU Session beyond providing (non-wildcarded) 5-tuples. Solutions fall into one of the following two categories:

-	Charging separation-only: Only the application detection filters in the UPF are provisioned with either IP Packet Filter Set (PFS) or PFD parameters,

-	QoS separation: The application detection filters in the UE and in the UPF are provisioned with either IP Packet Filter Set or PFD parameters in order to mark packets with the appropriate QFI inside the 5G System.

NOTE 1:	Both types of solution may also be used for traffic policing.

NOTE 2:	In context of the following candidate solutions to the Traffic Identification key issue, only the leading 6 bits of the Type of Service (ToS) field are considered. In case of Differentiated Services [D], these bits convey the DS field, as described in clause 5.3.2.2.

#### 5.3.6.2	Candidate IP-PFS Solution 1: Using IP ToS marking for downlink-only QoS flow mapping

This candidate solution focuses on a scenario where only downlink traffic needs to be mapped to a specific QoS Flow and handled differently by the 5G System. Related uplink traffic is handled using default QoS.

Editor’s Note: Such a solution is counterproductive for TCP- and QUIC-based transports, i.e. protocols depending on acknowledgements. Such solutions can make sense for RTP/UDP based flows, such as in Media Production.

#### 5.3.6.3	Candidate IP-PFS Solution 2: Using IP ToS marking for uplink-only QoS flow mapping

This candidate solution focuses on a scenario where only uplink traffic needs to be mapped to a specific QoS Flow and handled differently by the 5G System. Related downlink traffic is handled using default QoS.

Editor’s Note: Such a solution is counterproductive for TCP- and QUIC-based transports, i.e. protocols depending on acknowledgements. Such solutions can make sense for RTP/UDP based flows, such as in Media Production.

#### 5.3.6.4	Candidate IP-PFS Solution 3a: Using IP ToS marking for bi-directional QoS flow mapping, initiated by downlink traffic

This candidate solution focuses on a scenario where both downlink and uplink traffic for a particular application flow within a PDU Session shared by several different application flows (from the same or different UE applications) needs to be mapped to a specific QoS Flow and handled differently by the 5G System. The UPF is provisioned with a QoS Rule before the 5GMS-related application flow establishment. The UE QoS rule indicates the usage of reflective QoS. In this candidate solution, the 5GMS AF initiates the QoS Flow establishment by using specific ToS values in the downlink traffic.

Figure 5.3.6.4-1:

Assumptions:

-	A PCC rule for the UE is activate in the 5G System. The PCC rule contains a Service Data Flow Filter with a ToS value and the UE IP address.

-	Reflective QoS is enabled for the PDU Session in question (used here in step 3).

Steps:

Provisioning: The 5GMS System is provisioned for Dynamic Policy usage as defined in clause 5.7.2 of TS 26.501 [15]. As a result, various functions of the 5G System are provisioned for QoS usage as follows:

1.	The 5GMS Client has received Service Access Information (through M6 or M5), providing the information needed to use the Dynamic Policy Invocation API. Here, the sdfMethod indicates the usage of ToS. The 5GMS Client has activated a Dynamic Policy as described in clause 5.7 of TS 26.501 [15].

2.	The 5GMS AF has provisioned the information for a Dynamic PCC rule with the PCF (possibly through NEF).

3.	The PCF has authorized the request and created a PCC rule. The PCF has sent the PCC rule to the SMF, which has forwarded the QoS rule to the UE (indicating “reflective QoS” here) and to the UPF.

During media plane usage:

4.	The 5GMS Client initiates connection establishment by sending a TCP SYN packet. The packet is forwarded by the UE SDAP entity (Layer 2) and the UPF to the 5GMS AS.

5.	The 5GMS AS looks up the ToS policy, including the ToS value for this UE/network. Details are out of scope for 3GPP.

NOTE:	The 5GMS AS may also wait until the first HTTP request message is received to determine the purpose of the request. A 5GMS Client may use the TCP connection for subsequent HTTP transactions (persistent TCP connection).

6.	The 5GMS AS sends a TCP SYN–ACK to the UE to continue the TCP connection establishment handshake. The 5GMS AS sets the ToS field value. The packet reaches the UPF on its path to the UE.

7.	The UPF detects a PDR match for the UE. Here, the PDR for the UE IP address contains the ToS value. (The PDR was provided to the UPF in an earlier step as described in clause 5.3.4.3.)

8.	The UPF encapsulates the downlink IP packet inside an N3 packet. The UPF sets the QFI value in the N3 packet header.

9.	The UPF sends the N3 packet to the RAN and the RAN marks the QFI value in the SDAP layer, sending the packet to the UE.

10.	The UE SDAP entity (Layer 2) detects a new QFI.

11.	Reflective QoS is activated for the PDU Session and the UE creates a “UE-derived QoS Rule” as defined in TS 23.501 [23], clause 5.7.5.2.

12.	The UE SDAP entity (Layer 2) forwards the TCP SYN/ACK to the 5GMS Client.

13.	The 5GMS Client send the TCP ACK to complete the TCP connection handshake. (This packet does not need to be marked with a specific ToS value by the 5GMS Client.

14.	The UE SDAP entity (Layer 2) detects a PDR match for the UE. Here, the PDR is the 5-tuple as stored in the UE-derived QoS rule.

15.	The UE SDAP entity (Layer 2) encapsulates the IP packet containing the TCP ACK into the according radio protocols, including the QFI marking.

The 5GMS Client continues to use the established TCP connection.

Discussion:

-	The 5GMS AS needs to determine whether QoS should be used for this session and which ToS value to use.

-	The Npcf_PolicyAuthorization API allows a ToS value to be provisioned (without a direction indication), but the Nnef_AFsessionWithQOS API does not support provisioning of a ToS value.

-	The uplink traffic is not marked with a ToS field.

#### 5.3.6.5	Candidate IP-PFS Solution 3b: Using IP ToS marking for bi-directional QoS flow mapping, initiated by downlink traffic

This candidate solution focuses on a scenario where both downlink and uplink traffic for a particular application flow within a PDU Session shared by several different application flows (from the same or different UE applications) needs to be mapped to a specific QoS Flow and handled differently by the 5G System. In this candidate solution, the 5GMS AF initiates the QoS Flow establishment by using specific ToS values in the downlink traffic.

The difference between this and Solution 3a is that the UE receives a QoS rule containing an uplink Packet Filter with ToS.

Figure 5.3.6.5-1:

Assumptions:

-	A PCC rule for the UE is activate in the 5G System. The PCC rule contains a Service Data Flow Filter with a ToS value and the UE IP address.

Steps:

Provisioning: The 5GMS System is provisioned for Dynamic Policy usage as defined in clause 5.7.2 of TS 26.501 [15]. As result, various functions of the 5G System are provisioned for QoS usage as follows:

1.	The 5GMS Client has received Service Access Information (through M6 or M5), providing the information needed to use the Dynamic Policy Invokation API. Here, the sdfMethod indicates the usage of ToS. The 5GMS Client has activated a Dynamic Policy as described in clause 5.7 of TS 26.501 [15].

2.	The 5GMS AF has provisioned the information for a Dynamic PCC rule with the PCF (possibly through NEF).

3.	The PCF has authorized the request and created a PCC rule. The PCF has sent the PCC rule to the SMF, which has forwarded the QoS rule to the UE and to the UPF. The QoS Rule for the UE contains the ToS value.

During media plane usage:

4.	The 5GMS Client initiates connection establishment by sending a TCP SYN packet. The packet is forwarded by the UE SDAP entity (Layer 2) and the UPF to the 5GMS AS.

5.	The 5GMS AS looks up the ToS policy, including the ToS value for this UE/network.

NOTE:	The 5GMS AS may also wait until the first HTTP request message is received to determine the purpose of the request. A 5GMS Client may use the TCP connection for subsequent HTTP transactions (persistent TCP connection).

6.	The 5GMS AS sends a TCP SYN–ACK to the UE to continue the TCP connection establishment handshake. The 5GMS AS sets the ToS field value. The packet reaches the UPF on its path to the UE.

7.	The UPF detects a PDR match for the UE. Here, the PDR for the UE IP address contains the ToS value. (The PDR was provided to the UPF in an earlier step as described in clause 5.3.4.3.)

8.	The UPF encapsulates the downlink IP packet inside an N3 packet. The UPF sets the QFI value in the N3 packet header.

9.	The UPF sends the N3 packet to the RAN and the RAN marks the QFI value in the SDAP layer, sending the packet to the UE.

10.	The UE SDAP entity (Layer 2) forwards the TCP SYN–ACK to the 5GMS Client.

11.	The 5GMS Client send the TCP ACK to complete the TCP connection handshake. (This packet does not need to be marked with a specific ToS value by the 5GMS Client.

12.	The UE detects a QoS Rule match for the UE. Here, the IP Packet Filter contains only the ToS value.

13.	The UE SDAP entity (Layer 2) encapsulates the IP packet containing the TCP ACK into the according radio protocols, including the QFI marking.

The 5GMS Client continues to use the established TCP connection.

Discussion:

-	The 5GMS AS needs to determine whether QoS should be used for this session and which ToS value to use.

-	The Npcf_PolicyAuthorization API allows a ToS value to be provisioned (without a direction indication), but the Nnef_AFsessionWithQOS API does not support provisioning of a ToS value.

#### 5.3.6.6	Candidate IP-PFS Solution 4a: Using ToS marking for bi-directional QoS flow mapping, initiated by uplink traffic

This candidate solution focuses on a scenario where both downlink and uplink traffic for a particular application flow within a PDU Session shared by several application flows needs to be mapped to a specific QoS Flow and handled separated by the 5G System. In this candidate solution, the 5GMS Client initiates the QoS Flow establishment by using specific ToS values in the uplink traffic. Here, the reception of the ToS-marked IP Packet in the UPF triggers the creation of a new QoS rule in the UPF, similar to reflective QoS principles.

NOTE:	Creation of a new QoS rule derived from an IP packet is defined as “UE-derived QoS rule” creation in clause 5.3.4 (Reflective QoS) of TS 23.501 [15].

Figure 5.3.6.6-1:

Assumptions:

-	A PCC rule for the UE is activate in the 5G System. The PCC rule contains a Service Data Flow Filter with a ToS value and the UE IP address.

-	Reflective QoS is enabled for the PDU Session in question.

Steps:

Provisioning: The 5GMS System is provisioned for Dynamic Policy usage as defined in clause 5.7.2 of TS 26.501 [15]. As result, various functions of the 5G System are provisioned for QoS usage as follows:

1.	The 5GMS Client has received Service Access Information (through M6 or M5), providing the information needed to use the Dynamic Policy Invokation API. Here, the sdfMethod indicates the usage of ToS. The 5GMS Client has activated a Dynamic Policy as described in clause 5.7 of TS 26.501 [15].

2.	The 5GMS AF has provisioned the information for a Dynamic PCC rule with the PCF (possibly through NEF).

3.	The PCF has authorized the request and created a PCC rule. The PCF has sent the PCC rule to the SMF, which has forwarded the QoS rule to the UE and to the UPF.

During media plane usage

4.	The 5GMS Client initiates connection establishment by sending a TCP SYN packet. The packet is forwarded by the UE SDAP entity (Layer 2) and the UPF. The 5GMS Client has set a ToS value in the TCP SYN packet, as provided by the 5GMS AF in an earlier step (see clause 5.3.4.3).

5.	The UPF detects a PDR match for the UE. Here, the PDR for the PDU Session (e.g. identified by the TEID) contains the ToS value. (The PDR was provided to the UPF in an earlier step as described in Clause 5.3.4.3.)

6.	The UPF creates a “UPF derived QoS Rule”, similar to the “UE derived QoS Rule” (see TS 23.501 [15], clause 5.7.5.2). The UPF derives the IP Packet Filter set (similar to the derivation in the “UE derived QoS rule”) by taking the IP addresses, protocol identifier and port numbers into the IP Packet Filter Set.

7.	The UPF forwards the TCP SYN packet to the 5GMS AS.

8.	The 5GMS AS replies with a TCP SYN–ACK packet to continue the TCP connection establishment handshake.

9.	The UPF detects a PDR match for the UE. Here, the PDR for the UE contains the 5-tuple of the TCP connection.

10.	The UPF encapsulates the downlink IP packet into an N3 packet. The UPF sets the QFI value in the N3 packet header.

11.	The UPF sends the N3 packet to the UE via the RAN.

12.	The UE SDAP entity (Layer 2) detects a new QFI value.

13.	Since Reflective QoS is activated for the PDU Session, the UE creates a “UE-derived QoS Rule” as defined in TS 23.501 [23], clause 5.7.5.2.

14.	The UE SDAP entity (Layer 2) forwards the TCP SYN–ACK to the 5GMS Client.

15.	The 5GMS Client sends the TCP ACK to complete the TCP connection handshake. (Unlike in step 4, this packet does not need to be marked with a specific ToS value by the 5GMS Client.)

16.	The UE SDAP entity (Layer 2) detects a PDR match for the UE. Here, the PDR is the 5-tuple as stored in the UE derived QoS rule.

17.	The UE encapsulates the IP packet into the according radio protocols, including the QFI marking.

The 5GMS Client continues to use the established TCP connection.

Discussion:

-	TS 23.501 [23] defines only a “UE-derived QoS Rule”. The concept does not exist for the UPF.

-	The Npcf_PolicyAuthorization API allows a ToS value to be provisioned (without a direction indication), but the Nnef_AFsessionWithQOS API does not support provisioning of a ToS value.

#### 5.3.6.7	Candidate IP-PFS Solution 4b: Using ToS marking for bi-directional QoS flow mapping, initiated by uplink traffic

This candidate solution focuses on a scenario where both downlink and uplink traffic for a particular application flow within a PDU Session shared by several application flows needs to be mapped to a specific QoS Flow and handled separated by the 5G System. In this candidate solution, the 5GMS Client initiates the QoS Flow establishment by using specific ToS values in the uplink traffic. Here, the IP packet with the ToS value reaches the 5GMS AS and is re-used for downlink traffic.

Figure 5.3.6.7-1:

Assumptions:

-	A PCC rule for the UE is activate in the 5G System. The PCC rule contains a Service Data Flow Filter with a ToS value and the UE IP address.

-	Reflective QoS is enabled for the PDU Session in question.

Steps:

Provisioning: The 5GMS System is provisioned for Dynamic Policy usage as defined in clause 5.7.2 of TS 26.501 [15], Clause 5.7.2. As result, various functions of the 5G System are provisioned for QoS usage as follows:

1.	The 5GMS Client has received Service Access Information (through M6 or M5), providing the information needed to use the Dynamic Policy Invokation API. Here, the sdfMethod indicates the usage of ToS. The 5GMS Client has activated a Dynamic Policy as described in clause 5.7 of TS 26.501 [15].

2.	The 5GMS AF has provisioned the information for a Dynamic PCC rule with the PCF (possibly through NEF).

3.	The PCF has authorized the request and created a PCC rule. The PCF has sent the PCC rule to the SMF, which has forwarded the QoS rule to the UE and to the UPF.

During Media Plane usage

4.	The 5GMS Client initiates connection establishment by sending a TCP SYN packet. The packet is forwarded by the UE SDAP entity (Layer 2) and the UPF to the 5GMS AS. The 5GMS Client has set a ToS value in the TCP SYN packet, as provided by the 5GMS AF in an earlier step (see clause 5.3.4.3).

5.	The 5GMS AS reads the ToS value from the uplink packet. The 5GMS AS uses the uplink ToS value to mark all downlink packets in that TCP connection.

NOTE:	When the 5G System employs an N6 NAT, the N6 NAT may set the downlink ToS value to the same value as the uplink ToS value.

6.	The 5GMS AS sends a TYP SYN–ACK back to the UE. The packet reaches the UPF on its path to the UE.

7.	The UPF detects a PDR match for the UE. Here, the PDR for the UE IP address contains the ToS value. (The PDR was provided to the UPF in an earlier step as described in clause 5.3.4.3.)

8.	The UPF encapsulates the downlink IP packet into an N3 packet. The UPF sets the QFI value in the N3 packet header.

9.	The UPF sends the N3 packet to the RAN and the RAN marks the QFI value in the SDAP layer, sending the packet to the UE.

10.	The UE SDAP entity (Layer 2) detects a new QFI value.

11.	Since Reflective QoS is activated for the PDU Session, the UE SDAP entity (Layer 2) creates a “UE-derived QoS Rule” as defined in TS 23.501 [23], clause 5.7.5.2.

12.	The UE SDAP entity (Layer 2) forwards the TCP SYN–ACK to the 5GMS Client.

13.	The 5GMS Client send the TCP ACK to complete the TCP connection handshake. (Unlike in step 4, this packet does not need to be marked with a specific ToS value by the 5GMS Client.)

14.	The UE SDAP entity (Layer 2) detects a PDR match for the UE. Here, the PDR is the 5-tuple as stored in the UE-derived QoS rule.

15.	The UE SDAP entity (Layer 2) encapsulates the IP packet into the according radio protocols, including the QFI marking.

The 5GMS Client continues to use the established TCP connection.

Discussion:

-	The  5GMS AS needs to determine whether QoS should be used for this session and which ToS value to use.

-	The Npcf_PolicyAuthorization API allows a ToS value to be provisioned (without a direction indication), but the Nnef_AFsessionWithQOS API does not support provisioning of a ToS value.

#### 5.3.6.7	Candidate IP-PFS Solution 4c: Using ToS marking for bi-directional QoS flow mapping, initiated by uplink traffic

This candidate solution focuses on a scenario where both downlink and uplink traffic for a particular application flow within a PDU Session shared by several application flows needs to be mapped to a specific QoS Flow and handled separated by the 5G System. In this candidate solution, the 5GMS Client initiates the QoS Flow establishment by using specific ToS values in the uplink traffic. A ToS-based QoS rule is already provisioned, so that the Uplink Traffic is mapped to the correct QoS Flow.

Figure 5.3.6.7-1:

Assumptions:

-	A PCC rule for the UE is activate in the 5G System. The PCC rule contains a Service Data Flow Filter with a ToS value and the UE IP address.

-	A QoS rule with a ToS and a QFI value for uplink traffic is provisioned.

Steps:

Provisioning: The 5GMS System is provisioned for Dynamic Policy usage as defined in clause 5.7.2 of TS 26.501 [15], Clause 5.7.2. As result, various functions of the 5G System are provisioned for QoS usage as follows:

1.	The 5GMS Client has received Service Access Information (through M6 or M5), providing the information needed to use the Dynamic Policy Invokation API. Here, the sdfMethod indicates the usage of ToS. The 5GMS Client has activated a Dynamic Policy as described in clause 5.7 of TS 26.501 [15].

2.	The 5GMS AF has provisioned the information for a Dynamic PCC rule with the PCF (possibly through NEF).

3.	The PCF has authorized the request and created a PCC rule. The PCF has sent the PCC rule to the SMF, which has forwarded the QoS rule to the UE and to the UPF.

During Media Plane usage:

4.	The 5GMS Client initiates connection establishment by sending a TCP SYN packet. The 5GMS Client has set a ToS value in the TCP SYN packet, as provided by the 5GMS AF in an earlier step (see clause 5.3.4.3).

5. 	The UE SDAP entity detects a matching ToS value in the uplink traffic.

6.	The UE SDAP entity (Layer 2) encapsulates the IP packet into the according radio protocols, including the QFI marking

7.	The 5GMS AS reads the ToS value from the uplink packet. The 5GMS AS uses the uplink ToS value to mark all downlink packets in that TCP connection.

NOTE:	When the 5G System employs an N6 NAT, the N6 NAT may set the downlink ToS value to the same value as the uplink ToS value.

8.	The 5GMS AS sends a TYP SYN–ACK back to the UE. The packet reaches the UPF on its path to the UE.

9.	The UPF detects a PDR match for the UE. Here, the PDR for the UE IP address contains the ToS value. (The PDR was provided to the UPF in an earlier step as described in clause 5.3.4.3.)

10.	The UPF encapsulates the downlink IP packet into an N3 packet. The UPF sets the QFI value in the N3 packet header.

11.	The UPF sends the N3 packet to the RAN and the RAN marks the QFI value in the SDAP layer, sending the packet to the UE. The UE SDAP entity (Layer 2) forwards the TCP SYN–ACK to the 5GMS Client.

12. The 5GMS Client send the TCP ACK to complete the TCP connection handshake.

13.	The UE SDAP entity (Layer 2) detects a ToS match for the UE.

14.	The UE SDAP entity (Layer 2) encapsulates the IP packet into the according radio protocols, including the QFI marking.

The 5GMS Client continues to use the established TCP connection.

Discussion:

-	The 5GMS AS needs to determine whether QoS should be used for this session and which ToS value to use.

-	The Npcf_PolicyAuthorization API allows a ToS value to be provisioned (without a direction indication), but the Nnef_AFsessionWithQOS API does not support provisioning of a ToS value.

### 5.3.7	Conclusions

This Key Issue explored different solutions for traffic identification, so that the 5G System can detect different application flows and is enabled to apply an appropriate Policy and Charging Rules (PCC) to the associated traffic. The 5G System supports different means for direction-specific traffic detection. The use of the following traffic detection schemes is studied in detail in the context of 5G Downlink Media Streaming, e.g. assuming DASH content delivery:

-	5-tuple: UE IP address, server IP address, protocol, UE port and server port.

-	Type of Service (ToS) field: The ToS field is an 8-bit field within the IP header, that can be used to convey the 6-bit DSCP value. In IPv6, the Traffic Class field.

-	Packet Flow Description: 3-tuple of Server IP address, protocol and server port.

The use of the Flow Label (IPv6 only), the security parameter index (IPsec) and the domain name (part of the Packet Flow Description) are not studied in detail. It is recommended to study IPsec and the usage of the security parameter index for traffic identification is a subsequent study.

NOTE:	PFDs also support URL-based and domain name traffic detection. However, the URL is increasingly often encrypted (e.g. by the use of TLS or alternative encrypted transport) and therefore not visible to the network and domain name are also getting encrypted as encrypted forms of DNS and ECH are adopted.

Although TS 23.501 [23] defines a rich set of traffic detection schemes within the Packet Filter Set structure, only a subset of the schemes can be dynamically provisioned using APIs calls. Values for the ToS field, the Flow Label (IPv6 only), and the Security Parameter Index (IPsec only) can only be manually provisioned.

The data model of the Dynamic Policy API supports the selection of ToS, FlowLabel or Domain Names within the SdfMethod data type. However, there is no detailed definition on the usage of the feature, including the values for the ToS field. It is recommended to specify the usage of ToS and the configuration of the ToS values within a normative work item.

## 5.4	Additional/new transport protocols

### 5.4.1	Description

#### 5.4.1.1	General

Media streaming applications are continued to use HTTP-based distribution protocols, but newer versions of HTTP such as HTTP/2 or HTTP/3 are introduced, see for example also TR 26.925 [88], clause 6.1.4. The architectural and performance impacts of such protocols for 5G-based media distribution is unclear and requires study. The study also considers how Media Players may use functionalities existing in new transport protocols, and also investigate the impact of new transport protocols on 5GMS usage and traffic identification (e.g. Service Data Flow Descriptions).

Based on SMTP (Simple Mail Transport Protocol) [W] and reusing MIME (Multipurpose Internet Mail Extensions) notation [74][75], HTTP protocol (also known as web protocol), powers most websites, mobile apps, and videos. It was created by Tim Berners-Lee at CERN in 1989 and has been enhanced over the years to keep up with the ever-changing World Wide Web. Currently, the web is a mixture of HTTP/1.1 [3] and HTTP/2 [4] adoption. Most well-known websites are running HTTP/2, while smaller websites and late adopters plan to migrate to HTTP/2 in the near future as it is relatively easy to implement. HTTP/2 is used by about 45% of websites and supported by all major web browsers. HTTP/3 is only used by about 5% of websites now and not well-supported by web browsers yet. However. significant HTTP/3 deployments are emerging. For example, YouTube™ has for a long time been offering a pre-RFC draft version to any client that wants to use it, especially the Chrome™ browser. Other browsers are expected to follow soon after waiting for the QUIC and HTTP/3 RFCs to be published before mainlining that feature.

HTTP/2 introduces the "Streams" concept at HTTP level and each stream can have different priorities. All objects can from a web-page can be multiplexed in single long-lived TCP connection. Also, HTTP/uses header compression (HPACK) to avoid verbose/clear text. Also, HTTP/2 pseudo-mandates TLS to prevent “middle boxes” from messing up with the content. However, HTTP/2 does not remove the drawbacks of TCP’s head-of-line blocking - packet loss on one stream will block all other streams until recovery even if packets for all other streams are correctly received.

HTTP/2 testing shows [2] that the delivery of large objects over HTTP/2 can be slower than over HTTP/1.1 when there is packet loss. This is because HTTP/2 uses a single TCP connection, versus about six connections which most web browsers open over HTTP/1.1. In addition, the TCP congestion control algorithms reduce the TCP congestion window size, resulting in fewer bytes sent over the wire when using just one TCP connection.

HTTP/2 provides on average a 5% to 15% performance improvement on page load times over HTTP/1.1 [2]. HTTP/1.1 allows persistent TCP connections, but requests still had to be serialized, resulting in the well-known "HTTP head of queue blocking". In order to improve downloads, many TCP flows still needed to be parallelized to speed up delivery.

The solution to this problem is to use HTTP/2 over a different transport protocol that provides more efficient congestion control. One option would be to upgrade and modify TCP, but modifying TCP implementations is viewed as an impossible task. For example, middle boxes such as NAT, Firewalls, and Load balancers are problematic, because they get rarely upgraded which prevents any updates to TCP. TCP is also hard to evolve as it is almost always implemented as part of operating system kernels, requiring an updated operating system as part of TCP updates. Hence, it was considered easier to introduce a new transport protocol on top of UDP, that can be implemented outside the operating system kernel, in the user space. This new transport protocol is referred to as QUIC.

That, in essence, is what HTTP/3 [5] is: HTTP/2 over User Datagram Protocol (UDP) based on IETF QUIC. HTTP/3 is a thin layer on top of QUIC [32] including QPACK header compression [31]. The main QUIC functions are connection and stream multiplexing [32], fast startup[32], loss recovery, in-order delivery (within stream) [32], flow control [32]. TLS1.3 (handshake) [33], loss recovery and congestion control [34].

HTTP/3 always uses the QUIC protocol as its transport layer, although QUIC may also be used to carry other application-level protocols. For 5MBS, the term “HTTP/3” will always be used to refer to “HTTP/3 over QUIC”, unless the text refers specifically to QUIC in explaining its effect on HTTP/3.By multiplexing multiple concurrent logical streams over a single UDP-based transport association, and by giving each stream its own independent byte offset numbering space, packet loss in one stream does not block progress on other logical streams in the same QUIC connection. (However, the affected stream will still block when packets are lost, so as to guarantee in-order delivery of payloads to the application.).

Figure 5.4-1: HTTP/2 and HTTP/3 protocol stacks

For an entertaining introduction to QUIC and HTTP/3, please check https://www.youtube.com/watch?v=B1SQFjIXJtc.

However, using HTTP/3 over QUIC for adaptive streaming still requires study as under certain circumstances, the quality using QUIC may even degrade for DASH-based streaming than it would increase [6]. The evaluation results show that using the unmodified DASH algorithms on top of QUIC may not provide the anticipated performance boost when compared to the standard DASH over TCP.

The main expected benefit of QUIC is being able to multiplex requests for all Adaptation Sets onto the same transport association, and then to manage the network QoS on that aggregate connection. This has a valuable operational benefit to a CDN operator (including the 5GMS AS) in reducing the number of UDP ports that a server needs to keep open. Another benefit is being able to migrate connections from one IP address to another with minimal interruption to either client or server. This is useful when the client moves, but it is also useful when the server changes (e.g. in edge computing relocation Use Cases).

Because HTTP/3 and IETF QUIC are new protocols, there are several questions about performance and management that need to be investigated during this study.

#### 5.4.1.2	Performance Considerations for HTTP/3 over 5G Networks

The IETF specifications for HTTP/3 [5][31] and the core QUIC functions [32, 33, 34] are now approved in the IETF, and have been broadly deployed by a number of browser vendors and content providers, since the IETF QUIC working group has focused on specification, implementation, and, after the specifications were sufficiently stable, deployment, all in parallel. The performance of HTTP/3 in environments that have not been encountered during deployments to date is still an open question. Of greatest interest for this study, is the performance of HTTP/3 in 5G networks. Although deployment of 5G networks has begun, most deployment experience with HTTP/3 in mobile networks over the past few years has been in non-5G networks.

When end users have used HTTP/3 to access servers outside the 5G core network, 3GPP terminal mobility was handled transparently by the 3GPP network, and the UE’s IP address(es) didn’t change. If a server’s IP addresses changed, this was often not visible to the user, due to the widespread deployment of CDNs and load balancers in data centers. If edge computing resource IP addresses change in relocation use cases, QUIC connection migration could be used to reduce the impact on user experience, but this needs to be analyzed carefully.

MPEG-DASH has provided years of good user experience running over HTTP/1.1. As use cases arise which require very low latency, it is reasonable to better understand how, and when, MPEG-DASH might take advantage of HTTP/3 and analyze whether this has any implications for this study.

[6] raised the issue that MPEG-DASH performance might be lower over HTTP/3 than over HTTP/2. This reference was comparing Google’s pre-standardization QUIC implementation to highly optimized HTTP/2 over TCP implementations, and is about five years old, as of this writing, but the point remains – we need to know more about unmodified MPEG-DASH over standardized HTTP/3 implementations.

In particular, in the downlink media streaming Use Case, the server chooses the algorithm but the client media player decides whether to use a long-lived connection or to drop and reconnect when changing representation/rendition or adaptation/switching set, and the decision on connection (re)use is often delegated by the client application to a library, and this library might not offer an API to influence connection reuse. QUIC optimisations for reducing connection setup time (0-RTT or 1-RTT handshake (as described in [32]) mitigate this suboptimal use of connections to some extent, but the if the client, directly or indirectly by delegating the decision to the library, chooses to close and open connections, any QUIC congestion control algorithm begins probing for available bandwidth with no information about path characteristics. The connection handling characteristics of common APIs need to be analyzed as part of this study.

#### 5.4.1.3	Performance Considerations for IETF QUIC over 5G networks

The standardized QUIC congestion control and recovery procedures in [34] are chosen to emulate TCP’s standardized behaviours ([35], plus extensions). These are quite conservative, and not match current work on delay-based congestion control and recovery mechanisms, which have also seen wide deployment in QUIC implementations. In principle, delay-based congestion control and recovery mechanisms should improve user experience for streaming media applications, but this isn’t known yet, and this needs to be carefully analyzed. One of the core functions of QUIC is the capability to migrate connections without application involvement when endpoint IP addresses change, rather than requiring the detection of a connection failure, teardown of that connection, and setup of a new connection. Connection migration is one of the key QUIC functions that we do not have a great deal of experience with – implementers in the IETF said they were concentrating on performance for a connection, and many had not completely implemented or tested connection migrations at scale (data point is from October 2020).

#### 5.4.1.4	Management Considerations for HTTP/3 in 5G networks

One of the biggest distinctions between HTTP/2 over TCP and HTTP/3 over QUIC has been the encryption of almost all transport-level information carried in QUIC. This information, which was not encrypted in TCP even when it was carrying encrypted payloads, was often used in network management to identify and troubleshoot performance problems on the Internet. In most of experience with HTTP/3 deployments to date, content providers (e.g. Google/YouTube, Facebook, etc.) have terminated at least one end of the encrypted end-to-end connections, allowing them to identify problems at the QUIC transport level. That might be true in 5G deployments, or it might not be the case. If that is not the case, it would be very useful to consider the guidance in [36] as part of this study. It is also likely that operator deployments which relied on split-TCP connections to improve performance over radio links will require reconsideration for QUIC-based HTTP/3, since QUIC transport-level information is not available unless a device has a security context for the encrypted QUIC connection. It is likely that we will need to enhance the 3GPP QoS framework, and that if an application is using QUIC, 5-tuples are not sufficient for per-flow QoS.

Although QUIC can be implemented as part of operating system kernels, it is commonly implemented in user space, allowing frequent updates to congestion control and recovery procedures, including introduction of entirely new procedures (e.g. BBR, "Bottleneck Bandwidth and Round-trip propagation time" congestion control [37]). It is likely that the performance characteristics of HTTP/3 applications will vary more dynamically than HTTP/1.1 implementations that have been used by MPEG-DASH in the past. This may also have implications for existing applications if they migrate from MPEG-DASH over HTTP/1.1 or HTTP/2, to MPEG-DASH over HTTP/3.

#### 5.4.1.5	HTTP/3 client operation with an HTTP/3 server

There are many details involved, but the following description gives a sense of how little changes at the application layer when 5GMS begins using HTTP/3 as its application protocol.

1.	To open an HTTP/3 connection and retrieve a resource, an HTTP/3 client possesses a target URI providing a scheme, which needs to be "https:", an authority, which must include a resolvable DNS name, and (optionally) a path describing the location of the resource at the authority. This target URI may be obtained in various ways (configuration, dynamic lookup when the HTTP/3 client begins operation, or even from a resource that the HTTP/3 client has already retrieved).

2.	The location is resolved to an IP address, by looking up a DNS name, using HTTPS, or other mechanisms that would be used to resolve any other URI authority.

3.	Once in possession of the IP address for the HTTP/3 server, the HTTP/3 client attempts to open an HTTP/3 connection to the HTTP/3 server. In order to use HTTP/3, the HTTP/3 client communicates with the HTTP/3 server using the underlying QUIC protocol, sent over UDP. During connection establishment, the HTTP/3 client and HTTP/3 server perform a TLS 1.3 handshake, and HTTP/3 support is indicated by selecting the ALPN token "h3" in the TLS handshake. When the TLS 1.3 handshake is complete, both HTTP/3 client and HTTP/3 server have validated the connection.

4.	When connection validation is complete, the HTTP/3 client can begin performing normal HTTP requests over the HTTP/3 connection. In the case of M4d interactions, for example, the HTTP/3 client issues an HTTP GET request.

5.	When the HTTP/3 server receives an HTTP request, it responds using normal HTTP status codes, and then performs the requested operation if the request has succeeded.

#### 5.4.1.6	QLOG metrics reporting for HTTP/3 and QUIC

One adjustment to the 5GMS architecture to accommodate HTTP/3 could be to provide additional metrics reporting at Layer 7 and Layer 4. In addition to DASH application metrics, providing metrics on HTTP/3 protocol operation and, perhaps, even on QUIC might be useful for 5GMS System operators.

When DASH is used as a streaming protocol, DASH metrics would continue to be available when a DASH client creates an HTTP/3 connection to an HTTP/3 server.

When a non-DASH client, for example, an HLS [87] client, creates an HTTP/3 connection to an HTTP/3 server, a different mechanism would be necessary for metrics reporting. A capability called “QLOG”, currently under development in the QUIC working group of the Internet Engineering Task Force, is one such mechanism.

QLOG is composed of three related specifications:

-	a protocol-independent schema specification and mapping to JSON in [84]

-	a specification for HTTP/3-level events [85], and

-	a specification for QUIC-level events [86].

QLOG events can be stored, aggregated, and reported in a variety of ways. In particular, QLOG events can be collected at both endpoints of a connection, so these events could be reported by an HTTP client, an HTTP server, or both, if comparison is desired.

QLOG could be used to collect HTTP-level events for any HTTP-based streaming protocol running over HTTP/3.

QLOG could also be used to collect QUIC-level events for any streaming protocol encapsulated in QUIC, but consideration of this usage can be deferred until such protocols are identified.

### 5.4.2	Collaboration Scenarios

A service provider/content provider runs an adaptive media streaming service between HTTP/3 and QUIC enabled 5G Media Streaming AS and an HTTP/3 and QUIC enabled UE using 5G Media Streaming over M2d and M4d.

#### 5.4.2.1	General

The QUIC protocol [32] is a general-purpose transport protocol, although most current deployments have been in conjunction with HTTP/3 [5]. Even when limiting discussion to HTTP/3 over QUIC, this capability can be relevant to multiple key topics described in the present document, as well as the basic procedures for 5G Media Streaming described in TS 26.501 [15], in both downlink and uplink directions.

#### 5.4.2.2	HTTP/3 collaboration for downlink media streaming

For this key topic, the discussion will focus on the collaboration scenario where a 5GMS Application Provider runs an adaptive media streaming service between an HTTP/3-enabled 5GMSd AS and an HTTP/3-enabled 5GMSd Client using downlink media streaming over M2d and M4d. This is Collaboration Scenario 2, from clause A.2 in TS 26.501 [15], reproduced in figure 5.4.2-1 below.

Figure 5.4.2-1: Collaboration 2 (unchanged from Figure A.2-1 in TS 26.501 [15])

#### 5.4.2.3	Collaboration for uplink media streaming using HTTP/3

For this key topic, the discussion will focus on a media plane only collaboration scenario where the 5GMSu AS is deployed in the trusted domain. This collaboration scenario is described in clause 5.5.2.2 and illustrated in figure 5.5.2.2-1.

-	If HTTP/3 is used as the uplink application protocol at reference point M4u, the uplink media is streamed from the Media Streamer subfunction of an HTTP/3-enabled 5GMSu Client to an HTTP/3-enabled 5GMSu AS.

-	If HTTP/3 is used as the media egest application protocol at reference point M2u, the egest media is sent from an HTTP/3-enabled 5GMSu AS to an HTTP/3-enabled 5GMSu Application Provider.

Editor’s Note: Clause 5.5.2.7, describing a "hybrid, i.e. end-to-end form of collaboration across uplink media streaming and downlink media streaming services", and clause 5.2.4.3 on "Content preparation between uplink ingest and downlink streaming", would be useful to cover in clause 5.4 as well.

Editor’s Note: Uplink ingest formats are also a key topic in this study, and clause 5.2.1 names several existing uplink ingest formats to be studied. Even for the formats not routinely carried over HTTP (e.g. RTP-based RIST [21][22] and SRT [82]), discussions are underway in the Internet Engineering Task Force to specify direct mappings onto QUIC (for instance, [80] and [81] for RTP and [83] for SRT). If these uplink ingest formats are included in the final TR, a new 5.4.2.X clause should be added, describing the impact of encapsulating these non-HTTP protocols in QUIC.

### 5.4.3	Deployment Architectures

#### 5.4.3.1	General

A minimal deployment architecture that would provide HTTP/3 as transport for downlink and uplink media streaming is shown in figure 5.4.3.1-1.

Figure 5.4.3.1-1: Deployment Architecture for HTTP/3

The addition of HTTP/3 as a supported protocol at reference points M2 and M4 in the 5G media streaming architecture has little effect on the overall architecture. For example, a 5G System using HTTP/3 would resolve Fully-Qualified Domain Names (FQDNs) in the same way as with previous versions of HTTP, as described in annex B of TS 26.501 [15].

The biggest impact would be on implementations adjusting the endpoints they expose.

To use HTTP/3 for downlink or uplink media streaming, the 5GMS AS exposes HTTP/3 endpoints at reference points M2 and M4. This is in addition to the HTTP/1.1 and (optionally) HTTP/2 endpoints, as described in clause 6.2.1.2 of TS 26.512 [15].

In the following discussion, the term HTTP/3 client is used for the entity that initiates an HTTP/3 connection, and the term HTTP/3 server is used for the entity that the HTTP/3 client wishes to communicate with.

#### 5.4.3.2	HTTP/3 deployment in downlink media streaming

To use HTTP/3 for pull-based content ingest, the 5GMSd Application Provider exposes an HTTP/3-based origin endpoint to the 5GMSd AS at reference point M2d. This is in addition to the HTTP/1.1 and (optionally) HTTP/2 endpoints, as described in clause 6.2.1.2 of TS 26.512 [15]. Here, the 5GMS AS acts as an HTTP/3 client, the 5GMSd Application Provider acts as an HTTP/3 server, and HTTP operation proceeds as usual at M2d.

To use HTTP/3 for push-based content ingest, the 5GMSd AS exposes an HTTP/3-based origin endpoint to the 5GMSd Application Provider at reference point M2d. Here, the 5GMSd Application Provider acts as an HTTP/3 client, the 5GMS AS acts as an HTTP/3 server, and HTTP operation proceeds as usual at M2d.

To use HTTP/3 between the 5GMSd Client and the 5GMSd AS at reference point M4d, the 5GMSd Client acts as an HTTP/3 client, and the 5GMS AS acts as an HTTP/3 server, so that HTTP operation would proceeds as usual at M4d.

Because, as described in clause 6.2.1.2 of TS 26.512 [15], the 5GMSd Application Provider may use any supported HTTP protocol version for push-based content ingest at interface M2d, and the Media Stream Handler may use any supported HTTP protocol version at interface M4d, no other changes to the architecture are needed.

#### 5.4.3.3	HTTP/3 deployment in uplink media streaming

To use HTTP/3 for uplink media streaming, the 5GMSu AS exposes an HTTP/3-based endpoint to the 5GMSu Client at interface M4u. Here, the 5GMSu Client acts as an HTTP/3 client, the 5GMS AS would act as an HTTP/3 server, and HTTP operation proceeds as usual at M4u.

Separately, the 5GMSu AS may expose an HTTP/3-based endpoint to the 5GMSu Application Provider at reference point M2u for media egest. Here, the 5GMS AS acts as an HTTP/3 client, the 5GMSu Application Provider acts as an HTTP/3 server, and HTTP operation proceeds as usual at M2u.

#### 5.4.3.4	Usage of HTTP/3 at reference point M5

Editor’s Note: This section should be filled in as other key issues (clause 5.3 on traffic identification, clause 5.12 on network slicing) progress.

### 5.4.4	Mapping to 5G Media Streaming and High-Level Call Flows

#### 5.4.4.1	General

Using HTTP/3 for 5GMS functions that currently use earlier versions of HTTP (either HTTP/1.1 or HTTP 2/) can provide performance improvements, such as:

1.	“Single-threading” HTTP requests and responses – HTTP/1.1 doesn’t have any way of disambiguating response packets when multiple requests are outstanding on a connection, so the server sends responses as “next request received, next response returned”, even if the HTTP client has “pipelined” multiple requests without waiting for responses to its outstanding requests.

2.	Avoiding TCP “Head-of-Line” (“front-of-queue”) blocking – both HTTP/1.1 and HTTP/2 use TCP, so a missing response packet blocks all further response packets from being delivered to the application until the missing response packet has been detected and successfully retransmitted by TCP.

3.	Setting up connections in at most one “Round-Trip Time”, or RTT – HTTP/3 can set up a new connection in a single RTT. This is significantly faster than HTTP/1.1 or HTTP/2, because they use TCP, which requires a three-way TCP handshake, and then at least one more RTT to perform a TLS 1.3, and potentially multiple RTTs.

But beyond these performance improvements, there are very few impacts of adding the capability to use HTTP/3 to high-level call flows in 5GMS environments.

### 5.4.5	Potential open issues

#### 5.4.5.1	Introduction

Most of the potential questions previously identified in clause 5.4.1 don’t appear to be continuing concerns, as HTTP/3 has been deployed on the Internet at scale. A few open issues are worth noting here.

#### 5.4.5.2	Streaming Protocols taking advantage of HTTP/3 capabilities

Because so many media providers have used HTTP-based transport mechanisms, they have been forced to provide “workarounds” to overcome the TCP-related deficiencies that HTTP implementations were forced to deal with. Confirming with the predominant implementors of HLS, MPEG-DASH, SRT, and even RTP what their timeframes are for versions of these protocols that take advantage of HTTP/3 capabilities would be useful, but these versions are already starting to appear. Preliminary proposals for “Tunnelling SRT over QUIC” [SRT-QUIC], and multiple proposals for RTP over HTTP/3, or even directly over QUIC, have been submitted to the IETF.

#### 5.4.5.3	3GPP-specific impediments to HTTP/3 deployment

One well-recognized impediment to HTTP/3 deployment in the broader Internet is that it is carried over QUIC, which is a well-behaved transport protocol that detects and responds to path congestion, but QUIC runs over the UDP protocol, which is not understood to be well-behaved. Traditional UDP protocols have been query–response protocols, such as DNS domain name resolution ("what is the IP address for this domain name?" followed by "This is the IP address for that domain name"), and normally use well-known port numbers (if you send a UDP packet to port 53, network firewalls will assume that it is a DNS query, and unlikely to cause congestion).

HTTP/3, like any other protocol running over QUIC, will look like a highly encrypted UDP protocol, and it might use the UDP port number that matches the TCP port number of the same protocol running over TCP, but that is not a requirement, so network operators often investigate UDP packets being sent to an unfamiliar port number, especially if that traffic does not seem to be simple query-response traffic.

If their investigation is not reassuring, they may block UDP packets being sent to an unfamiliar port number at an unfamiliar IP address, and even if they don’t block that traffic, they may rate-limit the traffic to prevent their network links being overwhelmed by unknown traffic that might not respond to congestion indications. So, on the Internet, HTTP applications that attempt to use HTTP/3 are prepared to fall back to HTTP/2 or even HTTP/1.1 over TCP, which is more reassuring for network operators.

#### 5.4.5.4	Adaptive Streaming clients operating on top of HTTP/3 capabilities

Adaptive streaming clients are implemented to overcome the TCP-related deficiencies that HTTP implementations were forced to deal with. Today’s adaptive streaming clients are typically not aware that they are operating on top of HTTP/3 and QUIC instead of HTTP/1.1 and TCP. A DASH client as documented in clause 13 of TS 26.512 [?] includes some typical functions that may be impacted by operation on top of HTTP/3, in particular:

- 	Throughput estimation: An estimate of the network throughput from the 5GMSd Application Server, which is typically computed as the object size divided by the download time where the download time is the time difference between the first and the last bytes received for that object. According to [DASH-QUIC], measuring the throughput of multiplexed audio and video streams over a single UDP socket results in additional response latency for the (much smaller) audio segments, whose individual throughput is not captured during the calculation of channel throughput.

-	Request Scheduling: The adaptive streaming client schedules requests under the assumption of TCP operation. It typically operates for example audio and video on two HTTP sessions with separate sockets, and each of the sockets maintains its own independent socket buffer. By contrast, requests for audio and video are typically multiplexed onto a single HTTP/3 session on a single UDP socket with a single socket buffer. Therefore, the HTTP responses from both the streams interfere, and a higher response rate for one stream affects the queuing delay for the responses on the other stream.

Based on these operations, it is important that the adaptive streaming client:

1.	Is aware that it is operating on top of HTTP/3.

2.	Adapts its operation based on HTTP/3 properties

Details are for further study.

#### 5.4.5.5	5GMS Operation taking advantage of HTTP/3 capabilities

5G Media Streaming provides the ability to support regular OTT media streaming by providing additional and auxiliary information between the Media Session Handler and the 5GMSd AF. Supported functions in Rel-16 include telco CDN, network assistance and for example metrics reporting. It would be appropriate to adjust 5GMS function to HTTP/3 based delivery. As an example, certain DASH metrics are designed for TCP based streaming and would preferably be updated to account for HTTP/3 based delivery.

Editor’s Note: Identify the impact of including QLOG events on 5GMS metrics reporting.

#### 5.4.5.6	Addition of HTTP/3 as a supported HTTP version in 3GPP specifications

Clause 6.2.1.2 of TS 26.512 [15] describes the required and supported versions of HTTP for the 5GMS AF and 5GMS AS. For these functions to use HTTP/3, that clause must be updated, to add HTTP/3 as a supported HTTP version.

In addition to that update, it will be necessary to decide whether support for HTTP/3, and use of HTTP/3, should be required, encouraged, or permitted, over each of the relevant reference points.

Given our expectations for the pace of HTTP/3 deployment in 3GPP environments, our recommendation would be to permit HTTP/3 over most of the relevant reference points, in order to allow operators to plan for, and gain operational experience with, HTTP/3 usage in their environments.

#### 5.4.5.7	Pace of HTTP/3 deployment in 3GPP environments

HTTP/3 is ready at this time for use by some 3GPP operators with some types of 5GMS streaming applications, but the actual pace of deployment at any specific 3GPP operator will be based on

1.	The availability of HTTP/3 implementations on UEs, 5GMS AF instances, and 5GMS AS instances.

2.	The ability of operators and end users to see improvements in QoE with HTTP/3, which depends on the choice of streaming protocols in use, and support for HTTP/3 transport in those streaming protocols.

3.	The benefits of HTTP/3 on media plane interfaces (M2, M4) may realize more benefit than control plane interfaces (M1, M5), although, for example, there may be benefits from multiplexing M5 transactions into a single long-lived HTTP/3 session.

4.	The readiness of a 3GPP operator to support HTTP/3 and the underlying QUIC protocol, which may involve more than 5G Media Streaming. If the operator plans to use HTTP/3 in Service-Based Architecture, or plans to use QUIC in ATSSS, the operator may choose to deploy HTTP/3 for 5GMS with those other usages in mind.

### 5.4.6	Candidate Solutions

Editor’s Note: Provide candidate solutions (including call flows) for each of the identified issues.

#### 5.4.6.1	Metrics reporting using QLOG events

Editor’s Note: Contribution pending.

### 5.4.7	Conclusion and Recommendations

The study of this Key Issue has explored the ways in which the latest version of HTTP, HTTP/3, can be deployed to support the 5G Media Streaming architecture, and the potential open issues arising from this deployment.

1.	Because the architectural impact of moving from HTTP/1.1 or HTTP/2 to HTTP/3 on any 5GMS reference point is limited, this study has focused on minimal collaboration scenarios and deployment architectures for simplicity.

The study of this Key Issue makes the following recommendations:

1.	SA4 should begin work on updating 5GMS stage three specifications for reference points that currently use HTTP to allow the use of HTTP/3, to allow early deployment and to identify any unforeseen open issues with using HTTP/3 in a 5G System.

2.	At this time, support for HTTP/3 in 5GMS specifications, and usage of HTTP/3, should be allowed, but not required.

## 5.5	Uplink media streaming

### 5.5.1	Description

#### 5.5.1.1	Overview

Uplink media streaming functionality is currently under-specified in TS 26.501 [15] and TS 26.512 [16]. This part of the study investigates the gaps and potential solutions for completing the uplink streaming procedures, and associated protocols and APIs.

#### 5.5.1.2	Gap analysis of TS 26.501

The original focus of Rel-16 3GPP TS 26.501 [15] is on the overall system architecture, description of high-level procedures and call flows pertaining to downlink media streaming services.

In TS 26.501, there is significant imbalance in the scope and details of the described procedures for downlink vs. uplink media streaming. Key issues include the following:

1.	The procedures for downlink media streaming include session establishment, provisioning of various types of configuration information, metrics reporting, consumption reporting, dynamic policy and network assistance. In comparison, the described procedures for uplink media streaming are limited to session management, remote control, and network assistance. It is unclear for uplink streaming whether and how the configurations for content preparation, content protocols discovery, dynamic policy, metrics reporting, etc., should be defined and how the associated functions will operate.

-	Absent in clause 6 are procedural description and call flow regarding QoE metrics collection and reporting in uplink media streaming as compared to the presence of such text  in clause 5.5 on metrics collection and reporting in downlink media streaming.

2.	For downlink streaming, it is described that access to Service Access Information by the 5GMSd Client may be provided either over M8d by the Application Provider, or else fetched by the Client via M5u. For uplink streaming, the solely described method for the 5GMSu Client to obtain Service Access Information is via application metadata delivery over M8u. The only exception to this rule is the alternative method for provisioning Service Access Information to the 5GMSu Client by the 5GMSu AF, associated with remote control sessions in reference to remote control use cases and operational mechanisms in the context of FLUS (Framework for Live Uplink Streaming) as described in TR 26.939 [13] and TS 26.238 [14]. However, it should be noted that due to the limited description of the relationship between the uplink streaming framework and use cases, defined procedures and APIs in TS 26.512, it is unclear whether or how remote control sessions associated with uplink streaming delivery can make use of those interface procedures and APIs.

3.	Clause 4.3 is missing the description of Service Access Information parameters for a metrics configuration set pertaining to uplink media streaming, as compared to the presence of such information for downlink media streaming in clause 4.2.3. In particular, the “Metrics” parameter of the downlink streaming metrics configuration set is explicitly bound to the 3GPP “metrics” scheme and corresponds to one or more of the QoE metrics for either a progressive download or 3GP-DASH streaming service as defined in TS 26.247 [26]. On the other hand, given the lack of definition by SA4 of uplink streaming QoE metrics collection and reporting related functionality for existing application service specifications, there is no associated list of metrics on collection and reporting by the 5GMSu Client that can be referenced. Therefore, the following items are studied:

a.	Identify the quality metrics for uplink streaming.

b.	Identify the subset of metrics in a. that can be collected by UE and cannot be collected by the network entities such as the 5GMSu AF or the 5GMSu AS.

4.	For downlink streaming, the consumption reporting provides reports of the usage of download streaming of one single content.  However, any contribution reporting from UE for uplink streaming currently is not addressed. Therefore, the following items are studied:

a.	Identify the parameters useful for contribution reporting including the user’s initiated and interactivity events, as well as the user’s preferences for the uplink session such as hiding or reporting the location.

b.	Identify the subset of parameters in a. that can be collected by UE that cannot be collected by the network entities such as the 5GMSu AF or the 5GMSu for reporting the usage pattern to the Application Provider.

c.	Comparing the UE direct reporting of parameters (and metrics in item 3, above) to the collection of those parameters and metrics by the EVEX data collection AF.

#### 5.5.1.3	Gap analysis of TS 26.512

Similar to TS 26.512, the originally-defined features, protocols and APIs for 5GMS services mainly pertain to downlink media streaming services. More recently, as part of the specification maintenance process, additional descriptive, clarification and corrective text towards supporting uplink streaming services has been incorporated. However, remaining shortcoming include:

1.	The latter informationis centered on identifying relevance of existing and downlink streaming centric interface functionality for uplink streaming. For instance, provisioning and subsequent execution of content protocol discovery, dynamic policy invocation, and metrics reporting are regarded as applicable to support for uplink media streaming services. Also, content preparation via M1-based configuration of Content Preparation Templates may be considered a means for defining manipulations by a 5GMSu AS of media content uploaded from the 5GMSu Client. Such content manipulation may be associated with network-based media processing (NBMP) of uplink-delivered streaming media content (e.g., user-generated content of social media services, professionally-produced uplink streaming of sports events or concerts) as defined in FLUS [13] and further evaluated in the FS_FLUS_NBMP study item. However, further assessment of the potential linkage between content preparation and NBMP should be conducted.

2.	Although the Metrics Reporting Configuration resource as part of M1 provisioning procedures and described in clause 4.3.9 is intended to apply to either downlink or uplink media streaming, the corresponding data model of that resource (see clause 7.8.3) contains no explicitly-referenceable metrics for collection and reporting associated with uplink streaming services. This is due to the absence of a 3GPP-defined metrics scheme for uplink media streaming as it exists for downlink media streaming (e.g., the 3GPP scheme urn:‌3GPP:‌ns:‌PSS:‌DASH:‌QM10).

-	It would be desirable in this study to identify a candidate set of QoE metrics associated with uplink media streaming services, and as specified by a 3GPP-defined metrics scheme, to be collected by the 5GMSu Client and reported to the 5GMSu AF.

-	Up to now, in the development of the 5GMS architecture and associated protocols and APIs, very little attention has been given to defining control plane functionality that can offer unique/beneficial features to the end-user, network operator or Application Provider in the context of uplink media streaming service operation. Such value-added functionality could be enabled by leveraging the information available in the 5GMS network (e.g., subscription class, service characteristics, user/device mobility, network conditions) and which may be static or dynamic in nature. This is also an area that should be further studied.

#### 5.5.1.4	Gap analysis between TS 26.238 (FLUS) and TS 26.512 (5G Media Streaming)

TS 26.238 provides the following features:

1.	The FLUS Control Source may discover multiple FLUS sinks.

2.	The FLUS Control Source may discover the capabilities of each discovered FLUS Sink, including its network-based media processing capabilities.

3.	The FLUS Control Source may also request a FLUS Sink to perform media processing.

4.	The UE capabilities (formats, connectivity protocol, remote control) may be discovered by a FLUS Control Sink.

TS 26.512 uplink streaming currently lacks the above features.

### 5.5.2	Collaboration Scenarios

Editor’s Note: Study collaboration scenarios between the 5G System and Application Provider for each of the key topics.

#### 5.5.2.1	Overview

A set of key collaboration scenarios between an 5GMSu Application Provider and the 5G System Provider is described below.

Scenarios associated with Media Plane only collaboration and which may also involve downlink media distribution are presented first. Subsequently, scenarios pertain to both Control Plane and Media Plane collaboration and which may or may not involve downlink media distribution are presented.

NOTE:	In the collaboration scenario descriptions and diagrams below, an interface or API marked with a prime (′), e.g., M1′ or M2u′, denotes that while that interface/API functionally maps to its 3GPP-defined counterpart (e.g., M1 or M2u), its protocol and format are defined by the 5GMSu Application Provider. The implementation of these interfaces is up to the 5GMSu Application Provider discretion.

#### 5.5.2.2	Collaboration Scenario 1

This scenario pertains to a media plane only collaboration for which the 5GMSu AS is deployed in the trusted domain. Here, the 5GMS System Provider is assumed to offer uplink streaming capabilities as a service to an external 5GMSu Application Provider.

NOTE:	Although a Provisioning Session is shown in Figure 5.5.2.2-1 between the (external) 5GMSu Application Provider and the 5GMSu AF, due to the absence of the M5u interface in this diagram, there is no control plane collaboration between the 5GMSu Application Provider and the 5GMS System Provider.

Figure 5.5.2.2-1: Collaboration 1

#### 5.5.2.3	Collaboration Scenario 2

This scenario pertains to a media plane only collaboration for which the 5GMSu AS is deployed in the external domain and the 5GMSu AF is not involved. Specifically, the M1′ and/or M2u′ protocols do not follow 3GPP specifications.

Figure 5.5.2.2-1: Collaboration 2

#### 5.5.2.4	Collaboration Scenario 3

In this collaboration scenario, both the 5GMSu AS and 5GMSu AF are present. The 5GMSu AS resides in the external domain and does not employ 5GMS protocols and formats for uplink media reception from the 5GMSu Client, nor for content egest. The 5GMSu AF is used to interact with the 5G System, e.g., for dynamic policy invocation and/or other uplink streaming related network features such as metrics reporting and network assistance).

Figure 5.5.2.3-1: Collaboration 3

#### 5.5.2.5	Collaboration Scenario 4

In this collaboration scenario, both the 5GMSu AS and 5GMSu AF are present and follow 3GPP specifications. Both the 5GMSu AS and 5GMSu AF reside in the external DN/domain.

Figure 5.5.2.4-1: Collaboration 4

#### 5.5.2.6	Collaboration Scenario 5

This scenario is similar to collaboration scenario 4, with the exception that the 5GMSu AS and 5GMSu AF reside in the trusted DN/domain. An additional difference from collaboration scenario 4 is that the M2u API is used by the external 5GMSu Application Provider for content egest.

Figure 5.5.2.5-1: Collaboration 5

#### 5.5.2.7	Collaboration Scenario 6

This scenario represents a hybrid, i.e. end-to-end form of collaboration across uplink media streaming and downlink media streaming services. An external 5GMS Application Provider relies on the 5GMS System Provider to support both the uplink streaming media transmission by 5GMSu Clients and subsequent distribution of that content via downlink media streaming for reception by 5GMSd Clients.

Editor’s Note: The study on the key topic “Content Preparation” includes a use case whereby content preparation is used between uplink and downlink media streaming, and that use case can be considered as a specific example of this collaboration scenario.

### 5.5.3	Deployment Architectures

Editor’s Note: Based on the 5GMS Architecture, develop one or more deployment architectures that address the key topics and the collaboration models.

### 5.5.4	Mapping to 5G Media Streaming and High-Level Call Flows

#### 5.5.4.1	Collaboration scenario 1 call flow

Figure 5.5.4.1-1 provides a high-level call flow for the scenario depicted in clause 5.5.2.2.

Figure 5.5.4.1-1: Collaboration scenario 1 Call flow

Steps:

1.	The 5GMSu Application Provider creates a Provisioning Session for uplink streaming with the 5GMSu AF.

2.	The 5GMSu Application Provider creates a Content Publishing Configuration as part of the Provisioning Session that defines the instructions for content egest (M1u).

3.	The 5GMSu AF, based on the received Content Publishing Configuration, requests the 5GMSu AS to instantiate the content preparation process (M3u).

4.	The 5GMSu AS initialises the content preparation process.

5.	The 5GMSu AS acknowledges the initialisation of the required process (M3u).

NOTE:	M3u procedures between the 5GMS AF and the 5GMS AS are outside the scope of TS 26.512 [16].

6.	The 5GMSu AF acknowledges the successful creation of the Content Publishing Configuration to the 5GMSu Application Provider (M1u).

At some later point in time:

7.	The 5GMSu Application Provider optionally provides Service Access Information to the 5GMS-Aware Application (M8, out of scope).

8.	Uplink media streaming starts from the 5GMSu Client to the 5GMSu AS (M4u).

9.	Media streaming egest starts from the 5GMSu AS to the 5GMSu Application Provider (M2u).

Finally:

10.	The 5GMSu AS releases its resources after observing a period of inactivity.

NOTE:	Step 10 is implementation-dependent.

#### 5.5.4.2	Collaboration scenario 2 call flow

Figure 5.5.4.2-1 provides a high-level call flow for the scenario depicted in clause 5.5.2.3.

Figure 5.5.4.2-1: Collaboration scenario 2 Call flow

Steps:

1.	The 5GMSu Application Provider creates a Provisioning Session for uplink streaming with the 5GMSu AF (M1u′).

2.	The Provisioning function requests the 5GMSu AS to initialise the required content prepatation process (M3u′).

3.	The 5GMSu AS initialises the content preparation process.

4.	The 5GMSu AS acknowledges the initialisation of the required process (M3u′).

5.	The Provisioning function acknowledges the successful creation of the Provisioning Session to the 5GMSu Application Provider (M1u′).

At some later point in time:

6.	The 5GMSu Application Provider provides Service Access Information to the 5GMS-Aware Application (M8, out of scope).

7.	Uplink media streaming starts from the 5GMSu Client to the 5GMSu AS (M4u).

8.	Media streaming egest starts from the 5GMSu AS to the 5GMSu Application Provider (M2u).

Finally:

9.	The 5GMSu AS releases its resources after observing a period of inactivity.

NOTE:	Step 9 is implementation-dependent.

#### 5.5.4.3	Collaboration scenario 3 call flow

Figure 5.5.4.3-1: Collaboration scenario 3 Call flow

Steps:

1.	The 5GMSu Application Provider creates a Provisioning Session with its internal Provisioning function (M1u′).

2.	The Provisioning function requests the 5GMSu-like AS to initialise the required content preparation process instantiation (M3u′).

3.	The 5GMSu-like AS instantiates the content preparation process.

4.	The 5GMSu-like AS acknowledges the Provisioning the instantiation of required process (M3u′).

5.	The Provisioning function acknowledges successful provisioning to the 5GMSu Application Provider (M1u′).

6.	The 5GMSu Application Provider creates a Provisioning Session for uplink streaming with the 5GMSu AF.

X.	The 5GMSu Application Provider creates a Content Publishing Configuration as part of the Provisioning Session that defines the instructions for content egest (M1u).

At some later point in time:

7.	The 5GMSu Application Provider optionally provides Service Access Information to the 5GMS-Aware Application (M8, out of scope).

8.	The 5GMS-Aware Application requests the 5GMSu Client to start an uplink streaming session (M6u/M7u).

9.	The 5GMSu Client optionally (and in the case step 5 was not performed) requests Service Access Information from the 5GSMu AF (M5u).10.	Uplink media streaming starts from the 5GMSu Client to the 5GMSu-like AS (M4u′).

11.	Media streaming egest starts from the 5GMSu-like AS to the 5GMSu Application Provider (M2u′).

Finally:

12.	The 5GMSu AS releases its resources after observing a period of inactivity.

NOTE:	This step is implementation dependent.

#### 5.5.4.4	Collaboration scenario 4 call flow

Figure 5.5.4.4-1: Collaboration scenario 5 Call flow

Steps:

1.	The 5GMSu Application Provider creates a Provisioning Session for uplink streaming with the 5GMSu AF (M1u′).

2.	The 5GMSu Application Provider creates a Content Publishing Configuration as part of the Provisioning Session that defines the instructions for content egest (M1u′).

3.	The 5GMSu AF, based on the received publishing configuration, requests the 5GMSu AS to confirm the availability of content resources for egest (M3u).

NOTE:	M3u procedures between the 5GMS AF and the 5GMS AS are outside the scope of TS 26.512 [16].

4.	The 5GMSu AF acknowledges the successful creation of the Content Publishing Configuration to the 5GMSu Application Provider (M1u′).

At some later point in time:

5.  The 5GMSu Application Provider requests that the 5GMSu AF initialises the content preparation process (M1u′).

6.	The 5GMSd AF requests initialisation of the content preparation process (M3u).

7.	The 5GMSd AS initialises the content preparation process, if is not already running (M3u).

8.	The 5GMSd AS acknowledges the initialisation of the content preparation process (M3u).

9.  The 5GMSu AF acknowledges the initialisation of the cotent preparation process  (M1u′).

10.	The 5GMSu Application Provider provides Service Access Information to the 5GMS-Aware Application (M8, out of scope).

11. The 5GMS-Aware Application requests the 5GMSu Client to start an uplink streaming session (M6u/M7u).

Alternatively:

12.	The 5GMS-Aware Application requests the 5GMSu Client to start an uplink streaming session (M6u/M7u).

13.	The 5GMSu Client requests Service Access Information from the 5GSMu AF (M5u).

14.	The 5GMSd AF requests initialisation of the content preparation process (M3u).

15.	The 5GMSd AS initialises the content preparation process, if is not already running (M3u).

16.	The 5GMSd AF acknowledges the initialisation of the content preparation process (M3u).

17.	The 5GMSMu AF provides Service Access Information to the 5GMSu Client (M5u).

Then:

18.	Uplink media streaming starts from the 5GMSu Client to the 5GMSu AS (M4u).

19.	Media streaming egest starts from the 5GMSu AS to the 5GMSu Application Provider (M2u′).

Finally:

20.	The 5GMSu AS releases its resources after observing a period of inactivity.

NOTE:	This step is implementation-dependent.

#### 5.5.4.5	Collaboration scenario 5 call flow

Figure 5.5.4.5-1: Collaboration scenario 5 Call flow

Steps:

1.	The 5GMSu Application Provider creates a Provisioning Session with the 5GMSu AF.

2.	The 5GMSu Application Provider requests the 5GMSu AF to create one Content Publishing Configuration that defines the instructions for content egest (M1u).

3.	The 5GMSu AF, based on the received Content Publishing Configuration, requests the 5GMSu AS to confirm the availability of content resources for egest.

NOTE 1:	M3u procedures between the 5GMS AF and the 5GMS AS are outside the scope of TS 26.512 [?].

4.	The 5GMSu AF acknowledges to the 5GMSu Application Provider the successful creation of the Content Publishing Configuration (M1u).

At some later point in time:

5.	The 5GMSu Application Provider requests that the 5GMSu AF initialises the content preparation process (M1u).

6.	The 5GMSd AF requests initialisation of the content preparation process (M3u).

7.	The 5GMSd AS initialises the content preparation process, if is not already running (M3u).

8.	The 5GMSd AS acknowledges the initialisation of the content preparation process (M3u).

9.  The 5GMSu AF acknowledges the initialisation of the cotent preparation process  (M1u).

10.	The 5GMSu Application Provider provides Service Access Information to the 5GMS-Aware Application (M8, out of scope).

11. The 5GMS-Aware Application requests the 5GMSu Client to start an uplink streaming session (M6u/M7u).

Alternatively:

12.	The 5GMS-Aware Application requests the 5GMSu Client to start an uplink streaming session (M6u/M7u).

13.	The 5GMSu Client requests Service Access Information from the 5GSMu AF (M5u).

14.	The 5GMSd AF requests initialisation of the content preparation process (M3u).

15.	The 5GMSd AS initialises the content preparation process, if is not already running (M3u).

16.	The 5GMSd AF acknowledges the initialisation of the content preparation process (M3u).

17.	The 5GMSMu AF provides Service Access Information to the 5GMSu Client (M5u).

Then:

18.	Uplink media streaming starts from the 5GMSu Client to the 5GMSu AS (M4u).

19.	Media streaming egest starts from the 5GMSu AS to the 5GMSu Application Provider (M2u).

Finally:

20.	The 5GMSu AS releases its resources after observing a period of interactivity.

NOTE:		This step is implementation dependent.

As is shown, a new resource type, the Content Publishing Configuration is added. This describes the configuration of the egest (M2u) used in step 19.

#### 5.5.4.6	Collaboration scenario 6 call flow

The call flow for this collaboration scenario is described in 5.2.6.3.

### 5.5.5	Potential open issues

#### 5.5.5.1	Potential open issues in 5G Media Streaming stage 3

The following open issues seem to exist in TS 26.512 [16]:

1.	Lack of a standard template (or clear reference on how to use an existing standard template) for inclusion in a Content Publishing Configuration, i.e. to be able to provide content preparation instructions in a defined, interoperable format that the 5GMS AF supports through M1.

2.	Lack of definition of protocols for media egest from the 5GMSu AS to the 5GMSu Application Provider in uplink throughvia M2u.

NOTE:	The Content Protocols Discovery APIs allows the 5GMSu Application Provider to discover the supported egest protocols by 5GMSu AS. However, clause 8.1 of TS 26.512 does not currently list any specific egest protocols alongside those for downlink ingest streaming.

3.	Lack of content publishing API, i.e. a similar functionality to Content Hosting Configuration in downlink streaming, for provisioning the uplink streaming through M1u..

4.	Lack of Service Access Information for uplink streaming.

For downlink streaming, TS 26.512 [16] defines a StreamingAccess object as part of the Service‌Access‌Infromation resource. The StreamingAccess object includes a URL string that points to a media download resource or a manifest that describes a media presentation. In the case of uplink streaming, TS 26.512 does not yet specify which uplink streaming protocols are supported in M5u. Furthermore, it is not clear how the Media Session Handler would retrieve the entry point for uplink streaming to the 5GMSu AS.

#### 5.5.5.2	Potential open issues compared with FLUS

##### 5.5.5.2.1	General

Clause 5.5.1.3 describes the uplink streaming features from TS 26.238 [14] that are missing from TS 26.512 [16]. This section translates these missing FLUS features into potential new 5G Media Streaming features.

Table 5.5.5.2 shows a list of FLUS features and the equivalent features missing from TS 26.512. Note that in this table, the missing features of TS 26.512 are only listed for further discussion below, i.e. this is not a list of proposed features to be added.

Table 5.5.5.2-1: Mapping existing additional features of FLUS to 5GMS architecture

| Feature # | Existing support in FLUS | Equivalent in 5GMS | Needed or not? |
| --- | --- | --- | --- |
| 1 | The FLUS Control Source can discover multiple FLUS sinks. | The 5GMSu Client can discover multiple 5GMSu AS instances. | Supported by Edge Application Server (EAS) profile discovery as defined in TR 26.803 [Y] (see Discussion 1 below). |
| 2 | The FLUS Control Source can discover the capabilities of each discovered FLUS Sink, including its network-based media processing capabilities. | The UE5GMSu Client can discover the capabilities of each discovered 5GMSu AS. | Supported by EAS profile discovery (see Discussion 1 below). |
| 3 | The FLUS Control Source can also request a FLUS Sink to perform media processing. | The UE can also request the 5GMSu AS to perform media processing. | Not needed if the Content Preparation Template supports a generic media processing description such as NBMP (see Discussion 2 below). |
| 4 | The UE capabilities (formats, connectivity protocol, remote control) may be discovered by a FLUS Control Sink. | The 5GMSu Client capabilities may be discovered by 5GMSu AF. | Not needed in this form, since this information can be provided by 5GMS Application Provider (see Discussion 3 below). |

See the discussions below for further explanation.

##### 5.5.5.2.2	Discussion 1

The FLUS Discovery Server provides the means for a FLUS Control Source to discover multiple FLUS sinks and their capabilities. In the 5GMS architecture, various 5GMSd AS instances might have different capabilities. However, TS 26.512 does not provide a framework for describing 5GMS AS capabilities or any capability-based discovery mechanism.

TR 26.803 [46] proposes an edge-enabled 5GMS architecture for discovering EAS-enhanced 5GMSd AS instances and their capabilities by an Edge-Enabled Client (EEC) using EAS discovery filters. One possible way to discover 5GMSu AS capabilities and/or to instantiate a new 5GMSu AS with the desired capabilities is to use the procedure described in TS 26.803 [46] for the 5GMSu AS. This approach requires that the 5GMSu Client’s Media Session Handler supports the EEC logical function, the 5GMSu AF supports the EES logical function, and the 5GMSu AS supports the EAS logical function, as defined by TS 23.558 [47].

##### 5.5.5.2.3	Discussion 2

In some deployment scenarios, the request for media processing is performed by a FLUS Control Source by including a media processing document in its request to the FLUS Control Sink. Since in the present document content preparation is investigated for uplink streaming collaboration scenarios (clause 5.2.4.2 in the content preparation key topic), such functionality can also be used for media processing. If the content preparation template supports a generic media processing description framework such as NBMP, then content preparation can be used to provide equivalent functionality to media processing in the FLUS specification.

The 5GMS Content Preparation Template is provisioned through the M1 interface whereas in FLUS it is possible that the media processing is provisioned using the equivalent of the M5u interface.  To provide the UE with the ability to provision Content Preparation Templates, the following are possible options:

A.	The 5GMSu-Aware Application (if needed) provides the desired Content Preparation Template to the 5GMSu Application Provider via M8u and then the Application Provider requests provisioning of the Content Preparation Template through M1u, or

B.	The Media Session Handler in the 5GMSu Client requests the setting up of a Content Preparation Template by direct interaction with the 5GMSu AF via M5u. In this case, M5u needs to be extended to support Content Preparation Template provisioning requests from the Media Session Handler.

The current design supports option A. Option B seems unnecessary for the following reasons:

-	It wouldn’t be scalable to maintain a separate uplink streaming Provisioning Session at the 5GMSu AF for each and every UE, especially as the number of UEs becomes large.

-	In the current design, it is possible to create a separate Provisioning Session for each class of UE. In this approach, the UE signals its capabilities to the 5GMSu AF when requested Service Access Information at M5u, and then it is the task of 5GMSu AF to match the declared UE capabilities against the right uplink Provisioning Session metadata when responding to the request.

##### 5.5.5.2.4	Discussion 3

In the 5GMS architecture, the session is generally provisioned by the 5GMSu Application Provider. The Application Provider may already know the 5GMSu Client’s capabilities, for example through information in a user profile, or provided by the 5GMSu-Aware Application via M8u. Therefore, the need for the 5GMSu AF to discover the 5GMSu Client capabilities through M5u seems unnecessary.

### 5.5.6	Candidate Solutions

Since TS 26.512 [16] already defines solutions for the content ingest concerning the open issues of 5.5.5, one possible approach is to allow similar data structures, APIs, and protocols for content egest.

NOTE:	The candidate solution provided in the present document merely shows the desired features (by mirroring the distribution features) and is not intended as a proposed solution.

#### 5.5.6.1	Content egest protocols

The existing ingest protocols can be used for egest.

Table 5.5.6.1-1: Adding egest content protocols

| Description | Term identifier | Clause |
| --- | --- | --- |
| Content ingest protocols at interface M2d |  |  |
| HTTP pull-based content ingest protocol | urn:3gpp:5gms:content-protocol:http-pull-ingest | 8.2 |
| DASH-IF push-based content ingest protocol | urn:3gpp:5gms:content-protocol:dash-if-ingest | 8.3 |
| Content egest protocols at interface M2u |  |  |
| HTTP pull-based content ingest protocol | urn:3gpp:5gms:content-protocol:http-pull-ingest | 8.2 |
| DASH-IF push-based content ingest protocol | urn:3gpp:5gms:content-protocol:dash-if-ingest | 8.3 |

The highlighted rows indicate the added protocols.

#### 5.5.6.2 	Content Publishing Configuration API

An M1u API, similar to the M1d Content Hosting Configuration API as used for content ingest, can be defined for content egest.

Table 2: Operations supported by the Content Egest Configuration API

| Operation | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Create Content Publishing Configuration | content-publishing-configuration | POST | Used to create a Content Publishing Configuration resource. |
| Retrieve Content Publishing Configuration |  | GET | Used to retrieve an existing Content Publishing Configuration. |
| Update Content Egest Configuration |  | PUT, PATCH | Used to modify an existing Content Egest Configuration. |
| Delete Content Publishing Configuration |  | DELETE | Used to delete an existing Content Publishing Configuration. |
| Purge Content Publishing Configuration cache | content-publishing-configuration/purge | POST | This operation is used to invalidate some or all cached media resources associated with this Content Publishing Configuration. |

As is shown in the table, the sub-resource path in particular is changed for this resource, and other aspects remain identical to Content Hosting Configuration API.

#### 5.5.6.3 	Content Publishing Configuration Template

The Content Publishing Configuration resource, modelled after the Content Hosting Configuration resource, is shown in Table 5.5.6.3-1.

Table 5.5.6.3-1: Definition of Content Publishing Configuration resource

| Property name | Data Type | Cardinality | Description |
| --- | --- | --- | --- |
| name | String | 1..1 | A name for this Content Publishing Configuration. |
| PublishingConfiguration | Object | 1..1 | Describes the 5GMSu Application Provider's origin server to which media resources will be egested via interface M2u. |
| path | String | 1..1 | The relative path which will be used to address the media resources at interface M2u. This path is provided by the 5GMSu AF in the case of pull-based egest. |
| pull | Boolean | 1..1 | Indicates whether to the 5GMSu AS shall use Pull or Push for egesting the content. |
| protocol | URI String | 1..1 | A fully-qualified term identifier allocated in the name space urn:3gpp:5gms:content-protocol that identifies the content egest protocol. The set of supported protocols is defined in Table XXX. |
| entryPoint | String | 1..1 | An entry point to egest the content. The semantics of the entry point are dependent on the selected egest protocol. In the case of Pull ingest (pull flag is set to True), this parameter is returned by the 5GMSu AF to the 5GMSu Application Provider and indicates the entry point for pulling the content. In this case, the entryPoint shall be used as the base URL. In this case, the relative URL content address is provided out of band (e.g. with a manifest through M8u) to the 5GMSu Application Provider. In case of Push (pull flag is set to false), the entryPoint shall be provided to the 5GMSu AF to indicate the location to which content is to be pushed. In this case, the entryPoint shall be used as the base URL. |
| ContributionConfigurations | Object | 1..N | Specifies content preparation for the egested content. |
| contentPreparationTemplateId | String | 0..1 | Indicates that content preparation prior to egest is requested by the 5GMSu Application Provider. |
| canonicalDomainName | String | 1..1 | All resources of the upload shall be accessible through this default FQDN assigned by the 5GMSu AF. |
| certificateId | String | 0..1 | When content is distributed using TLS [16], the X.509 certificate for the origin domain is shared with the 5GMSd AF so that it can be presented by the 5GMSd AS in the TLS handshake at M2d. This attribute indicates the identifier of the certificate to use. |

Note that in the above table:

1.	The Pull mode is defined when the Application Service Provider pulls the content from the 5GMSu AS, and conversely, the Push mode is defined when the 5GMSu AS pushed the content to Application Service Provider.

2.	Each parameter description is updated based on item 1.

3.	Since in the Pull mode, the Application Service Provider needs to have the content URL addresses, it is assumed that that information is provided by other means (e.g. through M8u by the 5GMSu Aware Application).

4.	The DistributionConfigurations property in the Content Hosting Configuration resource for M1d is here replaced by the ContributionConfigurations property.

5.	The ContributionConfigurations.contentPreparationTemplateId property identifies the Content Preparation Template to be used. Like the Content Preparation Template resource as defined for downlink streaming in TS 26.512 [16], the data model of this resource is determined by its MIME content type.

#### 5.5.6.4	Uplink entry point

The StreamingAccess object in the Service Access Information resource can be extended to support uplink streaming entry points. This object may include the following information:

-	A URN, indicating an uplink streaming protocol provisioned for use over M4u (e.g. MPEG DASH, HLS, DASH-IF Ingest profile 1 or profile 2)

-	The entry URL for the above service (i.e. address of the 5GMSu AS for uplink streaming delivery by Media Streamer over M4u).

Additionally, the StreamingAccess property may include alternative media uplink streaming protocols for the same Provisioning Session by making it an array. For instance, two different entry points may be described in the Service Access Information for uplink streaming using DASH-IF Ingest profile 1 and profile 2.

An example of such extension is shown in the following table:

Table 5.5.6.4-1: Definition of ServiceAccessInformation resource
(based on an extract from clause 11.2.3.1 of TS 26.512 [16])

| Property name | Type | Cardinality | Usage | Description |
| --- | --- | --- | --- | --- |
| provisioningSessionId | String | 1..1 | RO | Unique identification of the M1d Provisioning Session. |
| … |  |  |  |  |
| StreamingAccess | Array(Object) | 1..1 | RO |  |
| mediaEntryType | Urn | 1..1 | RO | A fully-qualified term identifier from the controlled vocabulary urn:3gpp:5gms:content-protocol, as specified in clause 8, indicating the type of media service available at mediaEntry. |
| mediaPlayerEntry | Url | 1..1 | RO | For downlink streaming: Depending on the type of media entry indicated in mediaEntryType, either a pointer to a document that defines a media presentation (e.g. MPD for DASH content) that can be consumed via M4d, or else the URL of a media resource that can be streamed at M4d. For uplink streaming: Depending on the type of media entry indicated in mediaEntryType, either a URL endpoint on the 5GMSu AS to which media can be streamed directly at M4u, or else the URL of a document that can be downloaded from the 5GMSu AS which contains the parameters for uplink media streaming at M4u. |
| … |  |  |  |  |

### 5.5.7	Conclusion

This key issue has explored five collaboration scenarios for uplink streaming in the 5G Media Streaming architecture. For each scenario, a high-level call flow has been developed. Several gaps were identified during this process. Additionally, the uplink steaming features of TS 26.501 [15] and TS 26.512 [16] were compared with both TS 26.238 [14] as well as the download streaming features of TS 26.501 and TS 26.512, and the missing features are identified.

As this study shows, uplink streaming is severely underspecified in TS 26.501 and TS 26.512 in Release 16, and the following gaps are identified:

1.	Collaboration scenarios and their associated call flows described in clauses 5.5.2 and 5.5.3.

2.	Protocol(s) for egesting uplink streaming to the 5GMSu Application Provider through M2.

3.	Content Publishing Configuration API to enable a 5GMSu Application Provider to create, update, retrieve and delete a publishing template through M1.

4.	Content Publishing Template to describe the publishing configuration from the 5GMSu AS to the 5GMSu Application Provider, including content preparation prior to egest.

5.	Entry point for the UE’s uplink media streaming to the 5GMSu AS that is obtained by the UE through M5.

## 5.6	Background traffic

### 5.6.1	Description

Mobile Network Operators (MNO) are faced with the challenge of overload of their networks during peak hours. The following diagram shows a typical distribution of traffic over the day hours in a residential area:

Figure 5.6.1-1:	Example of traffic distribution over time

Note that the traffic distribution is also highly dependent on the geographic area. For instance, traffic distribution in a residential area maybe quite different from traffic distribution in a business or commercial area. Another example is traffic along roads during commute hours, which maybe higher by orders of magnitude than traffic during other hours.

As can be seen from the diagram, the traffic distribution is non-uniform/uneven throughout the day, which leads to congestion during the peak hours and very low utilization during off-peak hours. To alleviate this situation, the MNO may incentivize offloading traffic to off-peak hours to balance the network resource usage throught the day. The incentives may be provided in terms of preferential charging and guaranteed QoS.

### 5.6.2	Collaboration Scenarios

MNO and content provider enter an SLA that allows the content provider to distribute its content during off-peak hours to a set of receivers in a pre-determined geographic location.

Collaboration scenarios 2,4,5,7, and 8 from [TS256.501] are potential collaboration scenarios for traffic offloading using Background Data Transfer.

### 5.6.3	Deployment Architectures

There is no anticipated change to the 5G media streaming architecture to enable the usage of Background Data Transfer (BDT) for media distribution.

### 5.6.4	Mapping to 5G Media Streaming and High-Level Call Flows

The following is a potential high-level call flow for the configuration and usage of a BDT session in 5G media streaming:

Figure 5.6.4-1: Potential call flow for BDT session configuration and establishment

### 5.6.5	Potential open issues

The extensions to the client APIs, Provisioning API, and 5GMSd AF services to enable signaling and management of background data traffic sessions need to be specified.

### 5.6.6	Candidate Solutions

#### 5.6.6.1	Existing APIs to provision Background Data Transfer

The NEF offers the ResourceManagementOfBdt API, as defined in clause 5.4 of TS29.122 [44], to allow AF consumers to create, manage, and track Background Data Transfer (BDT) policies. The NEF also offers the ApplyingBdtPolicy API to request the application of a previously defined BDT policy for a particular UE or group of UEs.

The PCF offers the Npcf_BDTPolicyControl API, as defined in clause 5 of TS29.554 [55], to allow consumers to create and update BDT policies. The NF consumer may subscribe to notifications about any changes to the network conditions that affect a BDT policy.

A BDT policy consists of multiple transfer policies, each bounded by a time window in wallclock time. The 5GMS AF will select one of the transfer policies to apply.

#### 5.6.6.2	Potential Solution

Background Data Transfer is a generic feature that may be used by an Application Service Provider to pre-load content during time periods recommended by the MNO onto a UE. This feature can be suitable for multimedia streaming services but was developed to support other use-cases (such as automotive).

In this potential solution, the support for background data transfer in 5GMS can be realized by the following procedures:

1.	Provisioning by the 5GMS Application Provider to enable the usage of background data traffic and to define the background data transfer policy. The BDT policy may be part of the general QoS policy templates.

2.	Creation, updating, and monitoring the usage of the background data transfer policy in collaboration with the PCF. This may be done using the procedures described in 5.6.6.1.

3.	Communication of the background data transfer policy to the UE, e.g. as part of the access information.

4.	Registration by the 5GMS-Aware Application (via the Media Session Handler) with the 5GMS AF to use background data transfer.

5.	Notification by the 5GMS AF to the Media Session Handler of an opportunity to perform background data transfer.

6.	Activation of the background data transfer policy for the QoS flow, over which the transfer will happen.

Provisioning the background data transfer feature (step 1) is realized through an extension to the dynamic policy template as defined in clause 7.9.3 of TS 26.512 [16]. The dynamic policy template may, for example, be extended to include an additional property BdtReqData as defined in clause 5.6.2.3 of TS 29.554 [55].

The 5GMS AF uses the provisioning information to create or select a background data transfer policy within the PCF. For this purpose, it may use the Npcf_BDTPolicyControl or the ResourceManagementOfBdt API.

Once the policy is created, future downlink media streaming sessions will be able to use the defined background data transfer policy. The 5GMS AF provides the information about the background data transfer policy to the UE as part of the Service Access Information at reference point M5.

The M6 interface is extended to allow the application to request background download of content from the Media Session Handler. As a result, the Media Session Handler registers with the 5GMS AF to receive notifications about the availablity of a background data transfer opportunity. Once a download opportunity manifests itself, the 5GMS AF notifies the Media Sesssion Handler about the time window available for background content download, and the Media Session Handler will either perform the download itself, or else invoke the application to download the content. In case the MSH performs the download, appropriate extensions to the M6 API would be required to manage the access to and notification about completed/aborted download operations.

NOTE:	Integration of background data transfer with 5MBS is for future study. Alignment with the xMB API should be considered as part of this.

### 5.6.7	Conclusion

Background Data Transfer (BDT) is a feature that is offered by the 5G System to allow for data transfer during specific time windows with pre-arranged QoS and charging policies. Both PCF and NEF offer the required APIs to configure and use BDT. The discussion and the potential solution lay out the necessary extensions to integrate BDT as part of 5GMS procedures.

It is recommended that normative work is started to address the following objectives:

1.	Define the necessary extensions to 5GMS stage 2 and stage 3 to define the procedures and APIs to enable the use of Background Data Transfer in 5GMS, including:

a)	Provisioning of BDT for content hosting and distribution through M1

b)	Configuration of BDT and subscribe/notification for transfer opportunities and time windows through M5.

c)	Usage of BDT by the application through M6.

2.	Liaise with other relevant 3GPP groups to align on the usage of the BDT feature.

## 5.7	Content-Aware Streaming

### 5.7.1	Description

Content-Aware Encoding and statistical multiplexing of services are important and relevant technologies in the media industry. The impacts and opportunities of such technologies for 5GMS is not fully understood and requires study. For example, the currently-defined 5GMSd Application Function (AF) based network assistance solution is exclusively triggered by the Media Player, which instructs the Media Session Handler to interact with the Network. It might be more efficient for such network assistance functionality to be obtained directly from the content provider based on dynamic content complexity. Greater interaction with the 5GMS Application Provider during the lifetime of a session should be studied.

According to [2], if one analyses, almost any movie or television show scene by scene, you’ll notice the content has varying needs in terms of its fundamental complexity. Scenes with a lot of action and detail need a lot more bits in order to hit a quality target, whereas other scenes—say, a newsreader delivering a monologue—can achieve the same quality target with a reduced number of bits.

As an example, a game sequences provided for XR Traffic was encoded with x265 over 1 minute in Figure 5.7.1-1. One can see that at the same quality, the number of bits required to represent the content can be quite different.

Figure 5.7.1-1: Bit rate and quality over time for an example sequence.
(Blue bits, red PSNR in dB × 100)

Ideally, to maintain quality, one wants the bit rate to vary over time to maintain consistent quality regardless of the complexity of the scene. Four different scene types may be considered, and they differ in complexity- easy, moderate, hard, and very hard to compress. The “very hard” content might be a panning shot over a crowd, a shot of confetti falling, or simply a scene with a lot of high motion. Scenes such as these require more bits to convert all the motion and detail into a high-quality output that can be decoded and recreated accurately. A moderate scene, perhaps a close-up of a car, or an easy scene, like a single person speaking with no camera movement, will require fewer bits to deliver the same quality target as the harder scenes. In order to most efficiently encode the entire video, ideally a rate control mode that allocates more bits to the complex scenes, and fewer bits to the easier ones.

Different rate control algorithms exist:

-	CBR: Constant-Bit Rate encoding keeps the bit rate at a constant level, but the quality fluctuates. In ancient systems such as MPEG-2 TS, this is even addressed by sending lots of filler data just to keep the pipe constant

-	VBR: Variable Bit Rate encoding following the principle from above to keep the quality constant. This is often also referred to as Content-Aware Encoding nowadays (CAE).

-	Capped VBR: in this case the basic idea is to ensure that you have a mix of the above, i.e. a certain bit rate is never exceeded, but in case the content does not need the data rate, less data is sent.

The below diagram attempts to address and show these issues, but is more confusing then helpful.

Figure 5.7-2: Capped VBR rate control encoding compared with CBR and VBR

Variance in bit rates for different users may also result on the device and the consumption model. For a smaller screen, quite likely quality and bit rate requirements can be lower than for example going to a large screen such as a 4K TV.

The 3GPP QoS model contradicts this, as typically resources and QoS parameters are assigned for a session and only GBR is addressed.

From a adaptive bitaret streaming perspective, this content model needs to also be viewed as part on the streaming model, as the complexity of the content may be addressed based on the buffer availability, and also the situation of the network needs to be studied.

1)	On-demand Streaming:

a.	Stationary streaming of C-VBR/CBR content: Typically one operates with receiver buffer levels of 5-30 seconds [check details in TS 26.512]. One tries to keep the buffer filled. As soon as your buffer drains below some threshold, the client triggers a down-switch to a more sustainable bit rate. Switching typically can happen at segment boundaries, for example every 2 seconds.

b.	Start-up and seek. In this case, one starts basically starts from an empty buffer. In order to have quick and stable start up and good quality right away, there may be a benefit to get a higher short-term bit rate from the network to fill the buffer quicker to at least the switching threshold as you would not start playback until the threshold is reached. This is a very instantaneous action and needs to be fulfilled instantanteously, at most after 1 second.

c.	Stationary streaming of VBR/CAE content: In this case you basically operate on buffers of 5-30 seconds as above. The client typically has a map of the bit rate over time profile. In this case the client knows how much bit rate it needs for the next 5-10 seconds in order to keep the buffer stable and it can provide this information in a continuous manner to the network. The network will then grant a certain bit rate. This aspect may be fulfilled with using existing 5GMS functionalities.

2)	Live Streaming and especially low-latency live streaming:

a.	General: In this case the buffer is something of duration 1-5 seconds, it can be kept really low for low-latency streaming. Typically, one operates e2e latency of 3-5 seconds, so the buffer in the client is low. In addition, the client does not know the exact bit rate of the content as it is produced on the fly. Switching can typically be done every 1–2 seconds.

b.	Stationary streaming CBR: the buffer is much more susceptible, and you may have a threshold of maybe 500 ms when the client needs a fast arriving Segment is not arriving fast enough. This aspect may be fulfilled with using existing 5GMS functionalities.

c.	Start-up is similar to on-demand streaming as your buffer is anyways low. So no difference.

d.	Yet another and probably the most interesting case is the live case, for which the content and each of Representations are VBR encoded, but more following the content complexity and VBR/CAE is done as shown below. The content complexity is not known in advance, but it needs to be provided on an content ingest interface to the network. In this case, the network should provision very fast and dynamically the bit rate if needed, but can relax.

There are many other cases, where content complexity and device characteristics need to be taken into account when addressing quality of service.

### 5.7.2	Collaboration Scenarios

In the following, difference collaboration scenarios are provided. In Figure 5.7-3, content is generated by a third-party content provider in different formats and configurations, taking into account for example:

1)	Different device types (resolution, frame rates, codecs).

2)	Different streams (target qualities and bit rates).

3)	Encoding parameters (CBR, VBR, etc.).

Figure 5.7-3: Content-Aware Streaming based on static parameters

The network and client can make use of this information in order to optimize the streaming.

In a second variant, not only static information is provided, but also dynamic information with the media stream. This data is provided from the content provider to the 5G Media Streaming system and the client.

Figure 5.7-4: Content-Aware Streaming based on dynamic parameters

The client can use the information for:

-	Optimizing its own quality.

-	Acting fairly in a way that it only requests higher bit rates when the content is more complex, but leaves remaining capacity to the community.

In an On-Demand service, the use of the information is client controlled, i.e. the network is unaware of content complexity.

-	Client downloads a description of the content variations and the associated quality initially.

-	Client now brokers with the network for an average bit rate, but ability to request higher bit rate when content is complex

-	DASH client includes logic to use the VBR options smartly to ask for “boosts” ahead of time when content is complex

Live Streaming and Ingest (network more actively included in streaming):

-	Encoder provides as early as possible indication that content for same quality is getting a complex.

-	Network uses this and identifies, if and how to fulfill this for the clients that request it.

-	To not confuse client throughput estimation, communication between network and client is necessary.

### 5.7.3	Deployment Architectures

Editor’s Note: Based on the 5GMS Architecture, develop one or more deployment architectures that address the key topics and the collaboration models.

### 5.7.4	Mapping to 5G Media Streaming and High-Level Call Flows

Editor’s Note: Map the key topics to basic functions and develop high-level call flows.

### 5.7.5	Potential open issues

Editor’s Note: Identify the issues that need to be solved.

### 5.7.6	Candidate Solutions

Editor’s Note: Provide candidate solutions (including call flows) for each of the identified issues.

## 5.8	Network Event usage

### 5.8.1	Description

#### 5.8.1.1	Events exposed by 5GMS AF

The 5GMS AF performs several critical support operations for media streaming sessions. It also is responsible for collecting information about the progress and status of media streaming sessions. This information may be of interest to the AP or to other NFs in the network.

Thee 5G architecture defines event exposure mechanisms by the AF to other NFs in the network. TS 23.501 [23] and TS 23.502 [24] define the stage 2 Exposure service that can be offered by the AF. In TS 29.517 [25], the stage 3 realization of the Event Exposure service is specified as a RESTful API.

The resource structure is replicated in the following figure for convenience:

An Event Consumer subscribes to an application event and provides a URL on which it desires to receive the related event notifications. Both periodic reporting and immediate event reporting options are available. Event Filter Information and Event Reporting Information definitions as specified in TS 23.502 [24] and TS 29.517 [25] are used by the Event Consumer to indicate the desired event parameters and method of reporting for the selected event set.

So far, the following AfEvents are defined:

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| SVC_EXPERIENCE | Indicates that the event subscribed is service experience data for an application. | ServiceExperience |
| UE_MOBILITY | Indicates that the event subscribed is UE mobility information. | UeMobility |
| UE_COMM | Indicates that the event subscribed is UE communication information. | UeCommunication |
| EXCEPTIONS | Indicates that the event subscribed is exceptions information. | Exceptions |

Additional AFEvents may be defined.

#### 5.8.1.2	Events consumed by 5GMS AF

The 5GMSd AF may subscribe for event notifications related to application sessions, and receive event notifications for these sessions. Currently, TS 26.512 [16] requires the 5GMSd AF to subscribe with the PCF for the following types of event notifications:

-	Service Data Flow QoS notification control;

-	Service Data Flow deactivation;

-	Resources allocation outcome.

Additional events of interest to the 5GMSd AF are for further study.

### 5.8.2	Collaboration Scenarios

The Application Provider (AP) is outsourcing part of its content hosting to the MNO. The AP makes use of the Provisioning APIs to configure its content distribution. The AP would like to track the usage of the network resources for the distribution of its content as well as the QoE for its mobile subscribers. At the same time, it may choose to limit access to this information by the MNO to protect the AP’s service interests and/or user’s privacy. The AP configures data collection from UEs and the 5GMSd AS(s) to identity the data to be collected as well as the permitted entities and access levels to that data. The 5GMSd AF triggers the data collection accordingly and uses the AF Event Exposure framework to notify Event consumers about collected data and events.

### 5.8.3	Deployment Architectures

The deployment architecture for the data collection and exposure by the 5GMSd AF is depicted by the following figure:

Figure 5.8.3-1: Provisioning and Exposure Architecture

### 5.8.4	Mapping to 5G Media Streaming and High-Level Call Flows

The following is a sample call flow of the operation of the data collection and exposure by the 5GMSd AF:

Figure 5.8.4-1: Call flow for event exposure

A description of the steps is as follows:

1.	Provision data collection: The Application Provider provisions the data collection by configuring the data that is to be collected for all streaming sessions of this Provisioning Session.

2.	Trigger data collection: The 5GMS AF determines the data that is to be collected from the UE and provides this configuration to the Media Session Handler.

3.	Trigger data collection: The 5GMS AF determines the data that is to be collected by the 5GMS AS and uses the M3 interface to configure the 5GMS AS.

4.	Subscribe-notify pattern – Subscribe: In this phase, Event Consumers subscribe for the reception of notifications from the 5GMS AF. This phase may happen at any time. Subscriptions may be updated and canceled.

5.	Consumption and QoE reports: The Media Session Handler collects logs and information and sends them to the 5GMS AF.

6.	Access logs: The 5GMS AS collects logs and sends the information to the 5GMS AF

7.	Data aggregation and filtering: In this phase, the 5GMS AF aggregates the information received in the data collection phase, determines for each Event consumer the allowed level of access, and provides the information to the Event Consumers.

8.	If an Event Consumer opted for the subscribe-notify pattern, the 5GMS AF sends a notify message to that Event Consumer.

9.	In case the Event Consumer opted for the Query pattern, by setting the immRep flag to true, the 5GMS AF sends a one-time response with the event information to that Event Consumer.

### 5.8.5	Potential open issues

The following events are expected to be defined:

	Consumption reporting: this event contains reports sent by the UE about the consumption of the service, in terms of streaming session establishment.

	Quality of Experience Metrics reporting: this event contains reports sent by the UE about the QoE of the streaming sessions.

	Network Assistance: this event contains reports collected by the AF on the usage of network assistance such as bitrate recommendation and bitrate boost requests that have been offered for a streaming session.

	QoS and Charging usage: this event contains reports about QoS and charging policy requests made by the 5GMS AF for a streaming session.

	Access logs: this event contains reports about the access to the streaming content that is hosted by the network.

The granularity of the access to this information depends on the authorized access level. For each of these events, the triggers for the data collection and the levels of access to the collected data need to be defined as well.

### 5.8.6	Candidate Solution

#### 5.8.6.1	UE Data Collection via Direct and Indirect Methods

The 5GMS AF is designed to be the recipient of various types of application event information specific to media streaming, such as consumption reports, metrics reports and AF-based network assistance interactions, from the 5GMS Client (its Media Session Handler) over the M5 interface. Such collection of UE data is referred to in SA2’s eNA_Ph2 work item (see TS 23.288 [48]) as “direct data collection”.

It is possible that data collection from the UE is accomplished by the M8 application layer interface between the 5GMS-Aware Application and the 5GMS Application Provider (or, more generically, the Application Service Provider), and in turn forwarded by the ASP to the 5GMS AF, for further exposure s event services to NF consumers. Such mechanism, also referred by SA2 in [48] as “indirect data collection”, is not specified in TS 26.501 [6] and TS 26.512 [7], mainly because M8 functionality is outside the scope of 5GMS, and additionally, there was no use case/requirement in the 5GMSA/5GMS3 work items to drive the related stage 2 and stage 3 definition. On the other hand, due to the outreach from SA2 for AF support of indirect data collection, as shown by Figure 5.8.6.1-1, at least from the standpoint of such UE data being specific to media streaming services, it would make sense to define the requisite interface (i.e., between the ASP and the 5GMS AF) to support indirect data collection.

Figure 5.8.6.1-1: Indirect UE data collection
(copied from TR 23.700-91 [8])

While in principle the M1 interface could be extended for the purpose of indirect data collection (as even suggested by SA2 in the above diagram), logically it would make more sense to define a separate (SBI-based) interface for such purpose. M1 is mainly intended to support provisioning of session-based media streaming services. Indirect UE data collection by the 5GMS AF from the ASP corresponds to a more peripheral or auxiliary feature and might be better served by defining a separate interface or API exposed by the 5GMS AF, for example R1, which is the interface name identified in the BBC et al. discussion paper [49], as shown in Figure 5.8.6.1-2.

Figure 5.8.6.1-2: Generic reference architecture for UE data collection and reporting
(copied from S4-210723 [4])

It can be seen from the above diagram that R1 represents more than simply the indirect data collection procedure (referred to in Figure 5.8.6.1-2 as “indirect reporting from ASP”), by including reporting provisioning and event subscription and event publication (event exposure, e.g., via notifications) to the ASP, as further described in [49].

Note that reporting-related provisioning may pertain to the configuration of rules to be applied by the AF in the processing (e.g., anonymization, normalization, filtering, aggregation) of the data it has collected via direct or indirect methods. Such processing rules may differ for data to be reported by the AF to the ASP versus the data to be reported by the AF (via event exposure) to other NF consumers such as the NWDAF. From that perspective, when the Data Collection AF is instantiated inside a 5GMS AF, the reporting provisioning/configuration function logically belongs to M1 as part of overall provisioning functionality for 5G Media Streaming. Such extension of M1 along with additional specification of R1 is also reflected in [49] as shown in Figure 5.8.6.1-3:

Figure 5.8.6.1-3: 5GMS instantiation of generic architecture for data collection and reporting
(copied from S4-210723 [49])

The details of R1 coupling to and decoupling from M1 should be further studied for each of the R1 functions, with particular regard for the associated procedures, resources and data structures, for example, via emulation of M1 mechanisms and/or reuse of those defined for the Naf Event Exposure service defined in TS 29.517 [25].

#### 5.8.6.2	AF Collection of CDN Access Logs

As indicated in TS 26.501, the 5GMS AS acts as a CDN server (e.g., edge server) in the hosting and delivery of streaming media content (i.e., of ingested/egested content in downlink/uplink streaming). The corresponding content hosting related information, i.e., CDN access logs, available at the 5GMS AS can be forwarded to the 5GMS AF for subsequent event exposure to consumer entities such as the NWDAF or the Application (Service) Provider. Doing so requires the specification of an interface between the 5GMS AS and 5GMS AF. Such interface is logically represented by M3, although this internal interface, intended for the exchange of content hosting related information, is not further described/specified in Rel-16 TS 26.501 and TS 26.512. As part of the present study, it would seem possible to define M3 for the transfer of CDN log information between these entities. However, as defined in TS 26.501, M3 is intended to represent an undefined interface between the 5GMS AF and 5GMS AS for the exchange of content hosting related information. Therefore, it would be more logical to define an SBI-based reporting interface across R3, as shown in Figure 5.8.6.1-3, for the transfer of CDN access logs.

#### 5.8.6.3		Candidate media-related information for Event Exposure

As indicated previously, the stage 2 and stage 3 specifications for the Naf_EventExposure service are provided by TS 23.502 [24] and TS 29.517 [25], respectively. Up through Release 16, the categories of UE data at the AF available for event information subscription by NF consumers are the following:

-	Service Experience information for an application;

-	UE mobility information;

-	UE communication information; and

-	Exceptions information.

Comparison of the above categories with QoE metrics (as defined in TS 26.247 [40] for progressive download and DASH streaming services) and service experience and UE communication information types eligible for Naf event exposure (as defined in TS 23.288 [48]) shows the only common information attribute to be throughput. “Average throughput” is a defined quality metric in TS 26.247 for both progressive download and 3GP-DASH services, and “Throughput” is a defined type of performance data from the AF in TS 23.288. However, it is not unreasonable to expect that other QoE metrics collected by the 5GMS AF (based on TS 26.247 definitions for progressive download and DASH streaming) will also be eligible for event exposure to the NWDAF. In particular, at SA2#144-e, a CR to Rel-17 TS 23.288 in S2-2103267 [50] was agreed which adds “QoE metrics” as an additional type of service data related to Service Experience information for subscription by the NWDAF from the AF. Although the identified QoE metrics in that CR references those defined by MTSI in TS 26.114 [51], a CR from Qualcomm in S2-2104496 [52] proposes the inclusion in TS 23.288 of QoE metrics defined for 3GP-DASH and progressive download (per TS 26.247 [40]), VR (per TS 26.118 [53]), MBMS (per TS 26.346 [54]) and 5GMS (per TS 26.512 [45]). Therefore, it is possible that the QoE metrics defined by the 5GMS architecture will be adopted as valid service data for NWDAF subscription to Naf_EventExposure services in Rel-17 TS 23.288 [48] and TS 29.517 [25].

The definition of application event types, data components and formats relating to media streaming should be coordinated by SA4 with SA2 and CT3 in the production of the associated stage 2 and stage 3 specifications of Naf_EventExposure services.

## 5.9	Per-application-authorization

### 5.9.1	Description

#### 5.9.1.1	General

Operation of certain 5GMSA and 5G System enabled services include an SLA between the Application Provider and the 5GMS System provider. Different solutions to enable per-application authorization should be studied. . “Per-application authorization” refers to scenarios where one or more 5GMS-Aware Applications are hosted on the same UE (e.g. a SmartPhone) and may access services only from the associated 5GMS Application Provider.

The 5G System provider may offer one common 5GMSd AF or dedicated 5GMSd AFs. In the later case, the one 5GMSd AF instance services only a single 5GMSd Application Provider.

A set of example collaboration scenarios are described in clause 5.9.2.

#### 5.9.2.2	OAuth 2.0

OAuth 2.0 [91][92] is commonly used to provide authorization to RESTful HTTP APIs. OAuth separates the authorization server from the resource server. This typically simplifies deployments, since not every resource server such as an HTTP server needs to also provide authorization functionality. The separation of the authorization server from the resource server allows reusing access credentials for different services or between different parties.

The OAuth 2.0 architecture defines four main roles:

-	Resource Owner: A user or entity which can authorize access to a resource.

-	Resource Server: A function hosting the (protected) resource.

-	Client: A function which desires to access the resource hosted on the resource server.

-	Authorization Server: A function which authenticates the resource owner and provides different types of access tokens.

The roles can be visualized by the following simple example:

EXAMPLE:	A user stores some personal photos on cloud storage. When the user wants to print a photo using an external print service, rather than downloading the photo onto a local drive and then uploading the photo to the print service, the user can instead authorize the print service to fetch the photo from the protoected cloud storage. The Resource Owner here is the user. The Resource Server is the cloud storage. The print service is the Client.

The usage of OAuth 2.0 for the purpose of authorzing access to Network Function services is defined by SA3 in clause 13.4 of TS 33.501 [93].

In one possible 5G Media Streaming scenario, the 5GMS Application Provider subsidizes access to QoS and other network resources for its own applications. To ensure that only authorized 5GMS-Aware Applications are allowed to access these provisioned reousrces, the 5GMS Application Provider offers an authorization server (or configures such a server provided by the 5G System) that is used to authorize access to the Network Assistance feature of the 5GMS AF.

Here:

-	The 5GMS Application Provider acts as the Resource Owner, because it owns the Provisioning Session defining the QoS and charging policies.

-	The 5GMS AF acts as the Resource Server.

-	The Client is the Media Session Handler.

### 5.9.2	Collaboration Scenarios

#### 5.9.2.1	Collaboration A: UE hosting multiple Applications

This collaboration scenario focuses on cases where one or more 5GMSd-Aware Applications are hosted on the same UE and are using the same 5GMSd Client. This may be the case when the 5GMSd Client is provided as an Operating System level service. The 5GMSd Client supports isolation of the different 5GMSd-Aware Applications.

Figure 5.9.2-1: Per-Application Authorization Collaboration Scenario

Each 5GMSd-Aware Application uses an M8d reference point instance to connect to its 5GMSd Application Provider.

The 5G System provider offers a common 5GMSd AF within the trusted DN. The 5GMSd AF supports request and provider isolation so that 5GMSd Application Provider #1 and #2 are not interfering with each other. For example, 5GMSd Application Provider#1 has agreed different charging conditions than Provider #2 and the 5G System should ensure that only 5GMSd-Aware Application #1 can benefit from the conditions. Another example is different QoS levels, e.g. 5GMSd-Aware Application #1 is entitled to receive higher QoS than Application #2.

#### 5.9.2.2	Collaboration B: Applications with multiple subscription levels

This collaboration scenario focuses on cases where an Application Provider is offering multiple subscription levels to its consumers, for example 4K Premium or SD Standard QoS. This example is inspired by the use case from TS 26.512 [16], Annex A.2.

Figure 5.9.2-2: Per-Application Authorization Collaboration Scenario

Each 5GMSd-Aware Application uses an M8d reference point instance to connect to its 5GMSd Application Provider. The 5GMSd Application Provider is aware about the different subscription levels of the user.

The 5G System provider offers a common 5GMSd AF within the trusted DN. The 5GMSd AF needs to determine that 5GMSd Aware Application #1 is entitled to higher bit rates than 5GMSd-Aware Application #2.

### 5.9.3	Role distribution in the 5GMS deployments

The 5G Media Streaming architecture can be used for different application service offerings. Annex A in TS 26.512 [16] describes three different Dynamic Policy usage examples: Premium QoS, Conditional Zero Rating and Background Download. In all the three cases, different network features are used to realise the Dynamic Policy, e.g. an increase in network resource utilization when consuming HD content with the corresponding network QoS.

It is assumed in all three examples that the 5GMS Application Provider (and the Application Service Provider) has an agreement with the 5G System provider to use the relevant network feature.

Figure 5.9.3-1: Applying roles for 5G Media Streaming Architecture functions

Figure 5.9.2-1 illustrates the different roles and responsibilities:

-	The resource in question is a network policy.

-	The 5G System Provider is the resource owner in this case, since it provides the 5G connectivity service.

-	The 5GMSd-Aware Application is the Resource User. It instructs the 5GMSd Client to activate a certain dynamic policy, based on the service subscription and the selected content.

-	The 5GMS Application Provider is the Resource Owner. It checks that the requested dynamic policy matches the application service subscription. For example (with reference to clause A.2 in TS 26.512 [16]), when the user has an HD video subscription, the user should only be authorised to activate a dynamic policy corresponding to the HD operating point.

### 5.9.4	Mapping to 5G Media Streaming and High-Level Call Flows

Editor’s Note: Map the key topics to basic functions and develop high-level call flows.

### 5.9.5	Potential open issues

Editor’s Note: Identify the issues that need to be solved.

### 5.9.6	Candidate Solutions

#### 5.9.6.1	General

#### 5.9.6.2	Solution 1: Use of a Callback for authorization

This solution is based on the concept that the 5GMS AF can contact the 5GMS Application Provider (ASP) whenever a new Dynamic Policy is activated by a 5GMS Client.

The 5GMS Application Provider provides a different authorization token (e.g. a random number or a random string) via M8 to each 5GMS-Aware Application, so that each application can identify itself uniquely to the 5GMS AF.

When activating a Dynamic Policy, the 5GMS-Aware Application passes the token (via an M6 API call) to the Media Session Handler. When the Media Session Handler desires to activate a dynamic policy, it presents the authorization token to the 5GMS AF by invoking an M5 operation. Upon receit of such a token, the 5GMS AF executes a callback to the 5GMS Application Provider in order to verify, whether this authorization token is valid. When the token is valid, the UE application is authorized to activate this policy.

The authorization token is provided e.g. during the login procedure or is requested at a later stage. The UE Application may fetch metadata for the media assets at some stage.

The call flow is depicted below, assuming that the authorization token is provided with the application service login response.

Figure 5.9.6.2-1: Usage of a callback for policy activation authorization

The steps are as follows:

1.	When the user wants to use the 5GMS-Aware Application to consume e.g. video content, the user needs to authenticate with the application and the 5GMS Application Provider. (In some cases, this authorization can be cached/stored by the application, so that the user is not always challenged to provide the login credentials.)

NOTE:	The application may be a native application (e.g. an Android application) or a browser application.

2.	The 5GMS Application Provider determines the policy rights to which this application service subscription is entitled (e.g. the user may have subscribed to an SD quality video service or a 4K quality video service). According to the subscription entitlement level, the 5GMS Application Provider creates an authorization token and passes this token together with the login response back to the application.

3.	When the 5GMS-Aware Application (immediately or later) invokes the Media Session Handler to activate the network service from the 5GMS AF, the application passes the authorization token to the Media Session Handler. The authorization token can embed a user identifier, or the user identifier may be passed as separate (anonymised) parameter.

4.	When the Media Session Handler activates a dynamic policy, it provides the the token to the 5GMS AF, e.g. as an HTTP query parameter.

5.	The 5GMS AF then verifies the authorization token with the 5GMS Application Provider, using a callback function.

This callback URL can be stored by the 5GMS AF together with the Policy Template parameters so that the use of the network policy resource can be revalidated periodically with the 5GMSd Application Provider.

6.	When the 5GMS AF has verified that the 5GMS Aware Application is authorized to active the dynamic policy (based on the token), the 5GMS AF invokes the appropriate procuedres on the NEF or PCF. For example, the 5GMS AF triggers the addition of a QoS flow by invoking the Nnef_AFsessionWithQoS service.

#### 5.9.6.3	Solution 2: Time-limited authorization token provisioning

In order to reduce the number of callbacks, tokens with a limited validity duration may be provisioned with the 5GMS AF and the 5GMS-Aware Applications.

In this solution the 5GMSd Application Provider provisions a set of valid authorization tokens, including expiry time, in the 5GMS AF in advance via M1.

As in Solution 1, the Media Session Handler passes an authorization token when invoking the 5GMS AF at M5. The 5GMA AF authorizes the Media Sesssion Handler’s request based on this token.

Since the token validity is time-limited, the 5GMS Application Provider must periodically update the set of valid authorization tokens provisioned at the 5GMS AF and the 5GMS-Aware Application is responsible for refreshing the token used by the Media Session Handler. For example, the application may be configured to periodically fetch a new token from the 5GMS Application Provider.

Figure 5.9.6.3-1: Usage of time-limited tokens for policy activation authorization

The steps are as follows:

1.	The 5GMS Application Provider provisions tokens on the 5GMS AF before any 5GMS-Aware Application tries to activate any Dyanmic Policy. The token is provisioned together with the Policy Template definitions.

2.	When a user (and the 5GMS-Aware Application) successfully authenticates with the 5GMS Application Provider, the 5GMS-Aware Application receives a time-limited authorization token. The 5GMS-Aware Application typically stores the token.

The difference with Solution 1 is the use of the token by the 5GMS AF:

3.	When a 5GMS-Aware Application wishes to activate a Dynamic Policy, it provides the authorization token to the 5GMS AF. The 5GMS AF then validates the token using a simple lookup against it list of currently valid tokens without reference to the 5GMS Application Provider.

### 5.9.7	Conclusions

The Key Issue explores the usage of OAuth 2.0 for per-application authorization of different 5G System features. It is recommended to specify the usage of OAuth 2.0 (according to the SA3 guidelines) within a normative work item.

## 5.10	Support for encrypted and high-value content

### 5.10.1	Description

Content is increasingly encrypted for distribution for different reasons, e.g. Content Protection, Conditional Access, or integrity of playback. The management of keys for different use cases is a prime concern. Examples include scalable access to keys, secure storage of keys, key availabilities. It is envisioned that an MNO can provide key management and/or key distribution services for content providers. In particular, providing scalable and secure key management within 5GMS for multiple different devices needs further study.

Examples for secure media specification are for example provided by the MovieLabs ECP requirements and other content providers requirements.

In a specific example, a live sports service provider wants to offer a live stream. Examples include where the content needs to be delivered with low latency (typically encoder to glass in 3–10 seconds) in order to be on par with regular TV distribution means. Other services may also be considered.

The service may require different tools and functionalities levels of security:

1.	Conditional access supported by DRM management. As an example, users need to get a master key for decrypting the secondary level keys.

2.	Key rotation in order to support live streaming. As an example, these keys are changed periodically but protected by the master key.

3.	DRM and key management to ensure playback rules, for example to avoid that clients attempting early playback of the content too early and have advantages in betting/wagering, skipping content, etc.

4.	Watermarking. The content is distributed and a unique signature is added at the latest possible time (in the device, at the Edge). An example of such approach can be found here https://learn.akamai.com/en-us/webhelp/adaptive-media-delivery/adaptive-media-delivery-implementation-guide/GUID-3F89E64C-415D-452D-9541-BB650CD783B9.html.

5.	Content encryption.

6.	A secure implementation (use of TEE, Secure Media Path).

### 5.10.2	Collaboration Scenarios

It is assumed that the content provider provides DRM protections for the content. However, beyond this different collaboration models between the content provider and 5G System operator/MNO exist.

As examples, the MNO provides infrastructure to the content service provider in order to support security related functions.

-	The service provider may want to provide scalable access to the content and in particular the key distribution. Hence it uses 5G Media streaming servers to support secure key distribution.

-	The streaming service provider wants to rule playback, for example to avoid that the situation whereby users can see the streamed content too early while at the same time, the streaming service provider does not want to delay the distribution artificially either and want to give the clients the ability to download the main content (without buffer underruns).

-	The service provider asks for fairness in the client, but the client cannot be trusted to act fairly. Hacked clients are possible. Clients may have DRM systems that the service providers will use.

-	The service provider asks for a watermarking solution from the MNO.

Encryption (as already defined in TS 26.511 [3]) and secure keys may be used for other purposes, for example for conditional access or DRM systems. In some cases, keys are also provided in hierarchically, depending on business rules, security levels and deployment scenarios.

In an extension of the above use case, the content is distributed via multiple operators network. In this case, the encryption may be done by the service provider and the service provider provides the keys to the MNO. In another case, the service is offered by the MNO and the MNO does encryption and key management.

Editor’s Note: Study collaboration scenarios between the 5G System and Application Provider for each of the key topics.

### 5.10.3	Deployment Architectures

Editor’s Note: Based on the 5GMS Architecture, develop one or more deployment architectures that address the key topics and the collaboration models.

### 5.10.4	Mapping to 5G Media Streaming and High-Level Call Flows

Editor’s Note: Map the key topics to basic functions and develop high-level call flows.

### 5.10.5	Potential open issues

Editor’s Note: Identify the issues that need to be solved.

### 5.10.6	Candidate Solutions

Editor’s Note: Provide candidate solutions (including call flows) for each of the identified issues.

## 5.11	TV-grade mass distribution of unicast Live Services

### 5.11.1	Description

#### 5.11.1.1	General

Live TV services of different scale (professional, user-generated, session-based, etc.) are increasingly distributed over broadband and mobile networks. Live TV services are characterized by:

-	scalability (in terms of concurrent users),

- 	consistent quality,

-	high bandwidth requirements,

-	target latency constraints, and

-	TV Experiences.

#### 5.11.1.2	Scalability

Consistent support of the distribution of such services to a different scale of users and in a concurrent fashion is a prime concern. 5G Media Streaming is expected to support such service distribution and end-to-end optimizations. Improvements and optimizations on the architectural level and stage 3 are expected to be studied.

#### 5.11.1.3	Consistent quality

TV Services are expected to provide a consistent quality over time. Consistent quality can be defined as the service staying within a set of Operation Point boundaries to the largest extent. Specific aspects are:

-	Meeting the latency requirements, namely staying at the target, not exceeding the maximum, and not falling below the minimum. Measuring deviation from the target, as well creating events when exceeding the boundaries, is relevant. For more details see clause 5.11.15.

-	Meeting the target playback rate, i.e. how often the playback rate is changed from 1.0, and if there are cases when the playback rate is outside of the range. For more details see clause 5.11.15.

-	Staying within the boundaries of the target bit rates and meeting the target. This measurement includes those cases where the bit rate falls to 0, i.e. the service is stalled. All of this can be measured by the 5GMS Client.

Table 13.2.6-2 in TS 26.512 [16] defines a set of Operation Point parameters, repeated below, to which the Media Player is configured and that it exposes to the 5GMS-Aware Application as status information in notification events at reference point M7 when the current Operation Point changes. This configuration setting may be included in the media presentation manifest, or it may alternatively be provided through private application means. Generally, consistent quality refers to meeting the Operation Point parameters as shown in table 5.11.1.3-1 below . The Media Player and the network are expected to collaborate to meet the quality requirements.

Table 5.11.1.3-1: Operation Point Information (see TS 26.512, Table 13.2.6-2)

| OperationPoint |  |  | Operation Point Parameters | The currently configured operation point parameters according to which the DASH client is operating. |
| --- | --- | --- | --- | --- |
|  | Mode |  | Enum | The following operation modes are defined: live: The DASH client operates to maintain configured target latencies using playback rate adjustments and possibly resync. Vod: The DASH client operates without latency requirements and rebuffering may result in additional latencies |
|  | maxBufferTime |  | Integer | maximum buffer time in milliseconds for the service. |
|  | switchBufferTime |  | Integer | buffer time threshold below which the DASH clients attempts to switch Representations. |
|  | Latency |  |  | Defines the latency parameters used by the DASH client when operating in live mode. |
|  |  | Target | Integer | The target latency for the service in milliseconds. |
|  |  | Max | Integer | The maximum latency for the service in milliseconds. |
|  |  | Min | Integer | The maximum latency for the service in milliseconds. |
|  | PlaybackRate |  | MediaType audio, video, all | Defines the playback rate parameters used by the DASH client for catchup mode and deceleration to avoid buffer underruns and maintaining target latencies. |
|  |  | Max | Real | The maximum playback rate for the purposes of automatically adjusting playback latency and buffer occupancy during normal playback, where 1.0 is normal playback speed. |
|  |  | Min | Real | The minimum playback rate for the purposes of automatically adjusting playback latency and buffer occupancy during normal playback, where 1.0 is normal playback speed. |
|  | Bandwidth |  |  | Defines the operating bandwidth parameters used by the DASH client used for a specific media type or aggregated. The values are on IP level. |
|  |  | Target | Integer | The target bandwidth for the service in bit/s that the client is configured to consume. |
|  |  | Max | Integer | The maximum bandwidth for the service in bit/s that the client is configured to consume. |
|  |  | Min | Integer | The minimum bandwidth for the service in bit/s that the client is configured to consume. |
|  | PlayerSpecificParameters |  |  | Player specific parameters may be provided, for example about the used algorithm, etc. |

#### 5.11.1.4	High bandwidth requirements

TV Services typically have certain bit rate requirements that need to be met. Clause 5.1 of TR 26.925 [88] provides an overview. These figures are valid for both HDR/non-HDR video:

- 720p HD: 2 - 5 Mbps NOTE 1: Today typically 3 Mbps for HEVC [15] and 5Mbps for AVC [14], but bitrate reductions expected with better encoding and coding tools. See Clause 6.1.2. - Full HD: 3 - 12 Mbps NOTE 2: Today typically 5-7 Mbps for HEVC [15] and 10-12 Mbps for AVC [14], but bitrate reductions expected with better encoding and coding tools. See Clause 6.1.2. - 4k UHD: 5- 25Mbps NOTE 3: today typically 8-16 Mbps for HEVC [15] and 15-25 Mbps for AVC [14], but bitrate reductions expected with better encoding and coding tools. See Clause 6.1.2. - 8k UHD: 20 - 80 Mbps NOTE 4: Today typically up to 80 Mbps for HEVC [15], but bitrate reductions expected with better encoding and coding tools. See Clause 6.1.2. NOTE 5: On 8k UHD: currently not specified in 3GPP TS 26.116. Bitrate figures are based on limited available deployment data.

These figures may be used as indication of bit rate requirements for services.

#### 5.11.1.5	Target latency constraints

Based on a report developed jointly between DVB and DASH-IF on Low-Latency DASH [9], this clause defines details on how to support consistent latency in DASH for linear TV services. In [9], several definitions had been introduced, repeated here for consistency.

-	End-to-End Latency (EEL): The latency for an action that is captured by the camera until its visibility on the remote screen.

-	Encoder-Display Latency (EDL): The latency of the linear playout output (which typically serves as input to distribution encoder(s)) to the screen.

-	Packager-Display Latency: The latency after the output of the distribution encoder to the screen.

-	CDN latency: The delay caused by the CDN delivery from CDN input to CDN output.

-	Live Edge Start-up Delay (LSD): The time between a user action (service access or service join) and the time until the first media sample of the service is perceived by the user when joining at the live edge. Typically also the channel change time.

-	Seek Start-up Delay (SSD): The time between a user action (service access or service join) and the time until the first media sample of the service is perceived by the user when seeking to a time shift buffer.

Those two categories, latency and delay are subject to be controllable by the service provider for a consistent service offering. In the remainder, primarily the Encoder-Display Latency (EDL) and the Live Edge Start-up Delay are considered, but for some use cases also the End-to-End Latency (EEL) may be relevant. Figure 5.11.1-1 provides a schematic overview of the different latencies.

Figure 5.11.1-1: Different latencies and delays relevant for low-latency distribution

The Low Latency DASH scenario is a variant of the Live Services recommended approach focused on ensuring that the Encoder-Display Latency of the DASH Media Presentation is comparable to the latency when distributing over terrestrial, cable or satellite broadcast. Latency in broadcast is not a unique universal value, as it is influenced by many factors such as the duration of the broadcast encoding pipeline, the latency of the transport channel which can slightly differ per type (satellite, cable, IPTV or, DTT...), or the artificial delays introduced by local content moderation regulations. However, most of the measurements converge on a 3 - 10 seconds latency between the moment where the source signal is acquired for encoding and the moment when it's played back on the TVs, i.e the EDL. Start-up delay requirements are typically in the range of 1-2 seconds. For details refer to [9].

Low-latency mode are supported to minimize the architectural impacts on existing workflows. Figure 5.11.1-2 provides a basic flow of information for operating a low-latency DASH service as defined in DASH-IF’s Low-latency Modes for DASH [10]. The DASH packager gets information on the general description of the service as well as the encoder configuration. The encoder produces CMAF chunks and fragments. The chunks are mapped by the MPD packager onto Segments and provided to the network in incremental fashion using HTTP/1.1 chunked transfer.

Figure 5.11.1-2 Basic operation flow Low-Latency DASH

HTTP chunked transfer coding needs to be supported up from the ingest into the packager up to the CDN edge, whereas the last mile delivery is expected happen using HTTP chunked transfer coding or HTTP in regular mode. If HTTP chunked transfer coding is supported by the DASH player, it basically means that a media segment carrying the latest moment of the program (also known as the "live edge time" as defined in clause 4 of this document) could be consumed on the player while it's still being produced by the encoder and the packager.

In case chunked segments are used, clients may want to access partially available Segments for example for fast random access, see ISO/IEC 23009-1 [11]. However, requesting available byte ranges of a partially available Segment, i.e., Segments still being produced, is not consistently supported in CDNs, but solutions are provided in RFC8673 [X6]. This functionality may also be needed to support common segment handling for low-latency DASH and low-latency HLS.

Key aspects for low-latency live distribution include:

-	Consistent support for chunked transfer from ingest to client.

-	Support for partially access of non-complete resources.

-	End-to-end optimizations to support the latency requirements.

#### 5.11.1.6	TV Experiences

TR 26.917 [89] studied and identified requirements for TV Experiences. Clause 4.3 of TR 26.917 [89], reproduced below, provides a summary from a user-experience perspective:

- content delivery up to UHD resolution. - mechanisms to restrict the reception of some or all Subscribed TV services to groups of subscribers (e.g. based on the recipients of the services are subscribers of the MNO, roaming subscribers of other MNOs, or not subscribed to any MNO). - combinations of SD, FHD and UHD resolution TV transport services. - support network broadcast geographic area coverage management considering following criteria: - OTT provider request (including the potential coverage information of TV service information) - available network unicast/broadcast capacity of 3GPP network - number of users under broadcast network coverage - The location information of UE - support audio and video formats typically supported by TV Content Providers for SD and HD TV transport services and UHD TV transport services. - support codecs typically supported by TV Content Providers for HD TV services and UHD TV services. - support accessibility functions typically supported by TV Content Providers (e.g. subtitling, closed captioning, audio descriptions, anonymous reception, reporting to support ratings, reporting enforcement, etc.). - regulatory mandates typically supported by TV Content Providers (blackouts, emergency alerts, etc.). - interactivity functions typically supported by TV Content Providers (interactive services, second screen, personalization, etc.). - ad insertion use cases typically supported by TV Content Providers (targeted ad insertion, ad replacement, etc.), - encryption, security and conditional access functions typically supported by TV Content Providers. - concurrent delivery of multiple application components (TV service application signalling, statistical multiplexing, etc.). - random access and channel change times comparable to existing HD TV services.

#### 5.11.1.7	Summary of service requirements for Mass grade TV distribution

The following assumptions for the study are captured

-	Target for a maximum end-to-end service delay of 3 seconds (Packager-Display latency). This is measured from the input into the DASH Packager to the output of the DASH Player.

-	Target for a tune-in to an ongoing DASH Live stream of maximum 1.5 seconds (Live Edge Startup Delay). This is measured between the user action to tune-in (e.g. pressing a button) and the first frame is output from the DASH Player (still fulfilling the target of the Packager-Display Latency).

-	There is a requirement on a maximum ADG referring to the time difference between the first user to see a frame of media and the last user to see that same frame of media. It should be possible to remain below 1 second apart and within the target latency, i.e. max(ADG) = 1 second.

-	The DASH MPD can prepared so that the DASH Player can operate at different Latencies, e.g. low latency, normal latency or timeshift (video-on-demand profile).

-	TV Experiences as summarized in clause 5.11.1.6.

### 5.11.2	Deployment Architectures

#### 5.11.2.1	Distribution of low-latency media streams

A deployment architecture suitable for low-latency CMAF streaming is shown in Figure 5.11.2.1-1.

Figure 5.11.3-1 Deployment architecture for low-latency CMAF streaming

In this case:

1.	A live stream is ingested into a live encoder.

2.	The encoded stream is packaged into CMAF chunks.

3.	The packaged CMAF chunks are uploaded to an origin server using chunked transfer encoding input.

4.	Segments are then available for retrieval by a CDN on demand and moved through the CDN all the way to the client.

#### 5.11.2.2	Operation Point – Establishment and Monitoring

This clause deals with providing consistent quality in 5G Media Streaming as part of an Operation Point. Figure 5.11.2.2-1 shows how Operation Points and policies can be matched in a basic setup.

Figure 5.11.2.2-1 Operation Point workflow

The content-defined Operation Points are shown in the User Plane setup in Figure 5.11.2.2-1.

1.	A set of Policy Templates is agreed in advance between the 5GMS Application Provider and the 5G System operator. These may be provisioned in the 5GMS AF via reference point M1. Policy templates represent long term agreements made between the 5GMS Application Provider and the 5GMS System operator.

2.	Service Operation Points, defined in terms of the parameters reproduced in table 5.11.1.3-1, are long-lived profiles that will be used by media streaming sessions as references.

3.	Based on communication with the 5GMS-Aware Application, the UE device characteristics, the prevailing network conditions and so on, the Media Player selects a suitable Operation Point.

4.	Based on the parameters of the currently selected Operation Point, the policies in the 5G network are established, by the Media Session Handler instantiating one of the available Policy Templates by invoking the Dynamic Policies API at reference point M5. The streaming session uses at most one of the allowed policy templates at any point in time.

The above workflow is expected to be operational for 5G Media Streaming as defined today in TS 26.512 [15]. What is needed in order to execute the workflow is the following

1)	Well-defined Operation Points in the content. The Service Description as defined in ISO/IEC 23009-1 is mentioned in TS 26.512 [15] as a high-level statement:

The MPD may contain a one or several ServiceDescription elements that include operational parameters. The MPD may also include multiple configurations for the media (different codecs, different content protection, different resolutions, etc.), for example for playback under different operating policies. The handling of this information is documented in clause 13.2.

but not yet in TS 26.247 [40]. More details are needed, including some examples.

2) 	Media Session Handling APIs allowing a 5GMS-Aware Application to make use of dynamic policies, network assistance and metrics reporting in the 5GMS Client are not yet specified at reference point M6 in TS 26.512 [15]. In particular, clause 12.2 is incomplete for Dynamic Policy Information and Network Assistance information.

3)	Media Player metrics are not yet defined on the Media Streaming Handler API at reference point M7 defined in clause of TS 26.512 [15]. In particular, the extent to which the Media Player is (un)able to obey the currently selected Operation Point parameters needs to be signalled to the Media Session Handler in order to potentially drive the selection of a different Operation Point.

### 5.11.3	Collaboration Scenarios

#### 5.11.3.1	General

The following collaboration scenarios may be considered:

1)	Live content is provided to the MNO as an (uncompressed or mezzanine-compressed) contribution feed, and the MNO does the encoding and packaging for distribution.

2)	Live content is encoded and packaged by the content provider (for example as low-latency CMAF) and uploaded to the MNO. The MNO may produce an MPD for its distribution. However, the content provider augments the media with production and/or encoding timestamps (e.g. producer reference times) in order to permit latency measurements.

3)	The origin server is external to the MNO network and content is pulled through the 5GMS AS on demand by the clients.

#### 5.11.3.2	Distribution of low-latency media streams

For all of the collaboration scenarions described in clause 5.11.3.1 above, the content provider and the service provider agree on:

-	The MNO may monitor if the end-to-end latency target is maintained. This may for example be done by proper reporting.

-	The desired latency from glass-to-glass is met for example to be 3 seconds.

-	That the content is provided in low-latency, but also for consumption in time shift mode.

-	That the content can be accessed before the whole segment is uploaded.

### 5.11.4	Mapping to 5G Media Streaming and High-Level Call Flows

#### 5.11.4.1	General: Distribution of “Operation Point Services”

This clause provides an extension to the general call flow in clause 6.2.3 of TS 26.501 [15] in order to address operation point services.

Figure 5.11.4.1-1: High-level procedure for DASH content for Operation Point handling

Prerequisites:

-	The 5GMSd Application Provider has provisioned the 5G Media Streaming System and has set up content ingest.

-	The 5GMSd-Aware Application has received the Service Announcement from the 5GMSd Application Provider.

Extended Steps:

1:	Policy Templates are defined

12:	Media Player informs application about the current set of Operation Points.

13:	5GMSd-Awaer Application selects an Operation Point.

14:	Media Player provdes Operation Point parameters to the Media Session Handler.

15:	Media Session Hahdnler selects a Dynamic Policy based on the provided Operation Point parameters.

21: Media Player provides Operation Point metrics to the Media Session Handler.

22: Media Session Handler sends Operation Point measurements and events to the 5GMSd AF.

#### 5.11.4.2	Collaboration 1: MNO provides encoding and packaging

In this case, the specific aspects are as follows:

1)	A native stream is ingested into the network. This may be an MPEG-2 TS stream, an RTMP stream, etc. The stream may have time codes included that relate media time to real-time.

2)	Along with the ingest, a provisioning agreement between the content provider and the MNO exists on one or several Service Operation Points and Policy Templates. It may be that only one of the two is defined, and the other one is derived, or both are defined.

3)	The MNO encodes, packages and distributes the content according to the agreed Service Operation Points. The encoding and packaging may take into account the information from the Operation Points but may also take into account the information in the ingested media stream in order to control the latency.

4)	The Operation Point metrics collated by the 5GMS AF are used by the MNO to validate that the Service Operation Point is met, or to adjust the Policy Templates accordingly.

#### 5.11.4.3	Collaboration 2: MNO provides DASH distribution

In this case, the specific aspects are as follows:

1)	CMAF content is provided externally. The content is announced to the MNO for distribution.

2)	The ingest happens in a way such that the latency requirements can be met.

3)	A provisioning agreement between the content provider and the MNO exists on one or several Service Operation Points and Policy Templates. It may be that only one of the two is defined, and the other one is derived, or both are defined.

4)	The MNO distributes the content according to the agreed Service Operation Points, i.e. meeting bit rate and latency requirements. The MNO generates the media presentation manifests accordingly, for example the MPD and/or the HLS manifest.

5)	The Operation Point metrics collated by the 5GMS AF are used by the MNO to validate that the Service Operation Point is met, or to adjust the Policy Templates accordingly.

#### 5.11.4.4	Collaboration 3: MNO acts as CDN

In this case, the specific aspects are as follows:

1)	DASH or HLS content is provided externally. The content is announced to the MNO for distribution.

2)	The ingest happens in a way such that the latency requirements can be met.

3)	A provisioning agreement between the content provider and the MNO exists on one or several Servcie Operation Points and Policy Templates. It may be that only one of the two is defined, and the other one is derived, or both are defined.

4)	The MNO distributes the content according to the agreed Service Operation Points, i.e. meeting bit rate and latency requirements.

5)	The Operation Point metrics collated by the 5GMS AF are used by the MNO to validate that the Service Operation Point is met, or to adjust the Policy Templates accordingly.

#### 5.11.4.5	Operation Point example

An example for an operation point is provided below:

-	Mode = Live.

-	maxBufferTime = 4000.

-	switchBufferTime = 500.

-	Latency: target=3000, maximum=5000, minimum=3000.

-	PlaybackRate: max=1.2, min=0.8.

-	Bit rate: target = 4000000, max=6000000, min=1000000.

### 5.11.5	Potential open issues

The following aspects are not yet addressed in the current 5G Media Streaming specifications:

-	On M1d:

-	The establishment of consistent provisioning for Live TV services of different scale, providing the required Service Operation Points.

-	The ability to support Collaboration 1, for which the MNO provides encoding and packaging.

-	On M2d:

-	Ingest protocols that support chunk-based ingest and do not add latency by the use of chunked transfer. (No aggregation of chunks into resources.) Note, support for HTTP Chunked Transfer encoding is a requirement of HTTP 1.1 Clients and Servers. The gap is, that an 5GMS AS implementation is free to wait until the full segment is received.

-	Signaling of Operation Points along with the content in Collaboration 2.

-	On M4d:

-	Consistent support of low-latency streaming formats.

-	Time synchronisation between server and client so that performance (particularly latency) can be measured accurately.

-	Support for advanced TV Experiences.

-	On M5d:

- 	Metrics to monitor the operation of the service and if it meets operation, in particular the latency.

-	On M6d:

-	Dynamic policies, network assistance and metrics reporting.

-	Other:

-	Support TV service requirements.

### 5.11.6	Candidate Solutions

For the above open issues, the following candidate solutions are considered:

-	On M1d:

-	Policy Template updates to support TV services.

-	Provisioning extension to support Collaborations 1, 2 and 3.

-	On M2d:

-	DASH-IF Ingest Specification: https://dashif-documents.azurewebsites.net/Ingest/master/DASH-IF-Ingest.html

-	MPD extensions to support signalling of Operation Points using Service Description.

-	On M4d:

-	DASH-IF Low-Latency Extensions: https://dash-industry-forum.github.io/docs/CR-Low-Latency-Live-r8.pdf

-	UTC Time Sync that can be used by the 5GMS AS and the 5GMS Client in order to measure latency accurately. A 3GPP-based network time source may be provided.

-	New DASH and CMAF functionalities that support enhanced TV services, such as pre-selection.

-	On M5d:

-	Updates to DASH QoE metrics reporting for monitoring latency and Audience Drift Gap.

-	On M6d:

-	Extensions to M6 to address the requirements.

-	General support:

-	DASH-IF Low-Latency Extensions: https://dash-industry-forum.github.io/docs/CR-Low-Latency-Live-r8.pdf

### 5.11.7	Conclusions

Live TV services of different scale (professional, user-generated, session-based, etc.) are increasingly distributed over broadband and mobile networks, including 5G Networks. Live TV services are characterized by at least the following aspects: (1) scalability (in terms of concurrent users), (2) consistent quality, (3) high bandwidth requirements, (4) target latency constraints, and (5) advanced TV Experiences.

To address these type services a consistent support in the 5G Media Streaming Architecture, the protocols and codecs is needed. It is identified that the combination of low-latency CMAF formats, chunked transfer from content provider to the device, as well as consistent signaling and support of service quality are key aspects to the work.

It is recommended to support and optimize the deployment of unicast live TV services in 5G Systems. For this purpose, the following follow-up aspects are recommended to be addressed:

1.	Architectural and procedural considerations

a)	At least one call flow into that documents provisioning, ingest, distribution, presentation and monitoring aspects of low-latency live streaming services using CMAF Chunks.

b)	Updates to reference points to support provisioning, ingest, distribution, presentation and monitoring aspects of low-latency live services using CMAF Chunks.

c)	Typical configurable service parameters and operation points in terms of bit rates, latencies, Audience Drift Gaps, etc.

2.	Protocols, codecs, metrics and capabilities:

a)	Define capability mechanisms in order to identify the support of low-latency modes in 5GMS networks and clients

b)	Provisioning to support operation points and policy templates for low-latency live streaming.

c)	Create necessary extensions to support DASH and HLS chunked CMAF low-latency modes in an end-to-end workflow.

d)	Provide necessary protocols to scalably support time synchronization across 5GMS Applications, AS and 5GMS Clients (at appropriate precision).

e)	Extend QoE metrics schemes and metrics reporting functionality to address monitoring of Operation Point metrics for potential operational improvements.

f)	Provide extensions to formats and manifests support advanced TV experiences.

g)	Informative guidelines on using different Operation Points for low-latency live streaming.

## 5.12	Network Slicing Extensions for 5G Media Streaming

### 5.12.1	Description

#### 5.12.1.1	Overview

Network slicing standardization has progressed in 3GPP in sub working groups of both the SA and RAN groups. In particular, network slicing standardization that is relevant to our work in 3GPP SA4 has been specified in the SA2, SA5, SA6, and CT3 sub working groups.

3GPP SA2 specified network slicing related standardization in the following technical specifications:

-	TS 23.501 [23]: In this TS, SA2 specified network slicing concepts such as identification and selection of network slice (S-NSSAI and NSSAI), standardized SST values, Network slicing subscription aspects, UE NSSAI configuration and NSSAI storage aspects, network slicing support for roaming, interworking with EPS, network slice-specific authentication and authorization, network slice admission control etc.

-	TS 23.502 [24]: In this TS, SA2 specified procedures related to network slicing such as network slice-specific authentication and authorization procedure, network slice admission control function procedures, network slice admission control support for roaming, network slice admission control function (NSACF) services etc.

While 3GPP SA2 has specified architectural concepts about using network slices, 3GPP SA5 has specified the architecture, provisioning, and management (creation, modification, and termination) of network slices in addition to defining roles related to 5G networks and network slicing, and management models for network slicing. The network slicing related standardization specified by 3GPP SA5 can be found in the following technical specifications:

-	TS 28.530 [56]: Describes general concepts such as management of 5G networks and network slicing, principles of network slicing management framework, slice profile and service profiles, and business level requirements

-	TS 28.531 [57]:  Describes requirements such as creation, modification, activation, deactivation of network slice instances. network slice subnet instances, and 3GPP NF instances. Operations for management of network slice instances and network slice subnet instances is also specified.

-	TS 28.532 [58]: Describes management and orchestration concepts such as provisioning management services, fault supervision management services, and performance assurance management services. Corresponding stage-3 management service specification is also specified.

-	TS 28.533 [59]: Describes the architecture framework for management of network slicing including the architecture reference model for management interactions with NFV MANO, ZSM framework, and NWDAF

-	TS 28.540 [60]: Describes 5G Network Resource Model (NRM) for NR and NG-RAN specifying aspects related to requirements for management of network slice and network slice subnets.

-	TS 28.541 [61]: Describes stage-2 and stage-3 specification of 5G NRM including the information model definitions for network slice NRM such as NetworkSlice, NetworkSliceSubnet, ServiceProfile, and SliceProfile.

-	TS 28.542 [62]: Describes stage-1 NRM for 5G Core Network.

-	TS 28.543 [63]: Describes stage-2 and stage-3 NRM for 5G Core Network.

-	TS 28.545 [64]: Describes stage-1 fault supervision aspects about management and orchestration of networks and network slicing.

-	TS 28.546 [65]: Describes stage-2 and stage-3 fault supervision aspects about management and orchestration of networks and network slicing.

-	TS 28.552 [66]: Describes 5G performance measurements related to network slicing instances.

-	TS 28.554 [67]: Describes 5G end to end key performance indicators (KPIs) related to network slicing.

In addition to the SA2 and SA5 groups, 3GPP SA6 has specified network slicing related standardization in the following technical specifications:

-	TS 23.434 [68]: Specifies the functional architecture for service enabler architecture layer (SEAL) and the procedures, information flows, and APIs for each service within SEAL in order to support vertical applications over the 3GPP system. As part of this specification, aspects related to network slice capability management is specified including procedures and information flows for network slice capability management.

-	TR 23700-99 [69]: Proposes application architecture aspects solutions and enhancements to SEAL using the NSCALE application layer enablement.

Finally, some work related to network slicing has been done by the 3GPP CT3 group in the following technical specifications:

-	TS 29.520 [70]: Specifies the stage-3 definition of Network Data Analytics Function Services of the 5G system. Proposes the data model for network slice information that NWDAF can provide to authorized consumers.

#### 5.12.1.2	Network Slicing Extensions in SA4

Though the 3GPP SA4 technical specifications related to 5G Media Streaming have touched upon network slicing, the standards could be significantly enhanced with further study and specification related to network slicing aspects. Some of the items currently being worked in SA4 that can be enhanced to incorporate network slicing based specification are presented below. Table 5.12.1.2-1 lists the Release-16 architecture items that can benefit from further specification on network slicing.

Table 5.12.1.2-1: Release-16 Items for further Network Slicing related specification

| Work Items | Aspects for study related to network slicing |
| --- | --- |
| 5G Media Streaming | TS 26.501 [15] and TS 26.512 [16] have added specification text for dynamic policy. However, aspects related to network slicing and dynamic policy are not adequately addressed. TS 26.501 [15] briefly discusses dynamic policy based on network slicing for downlink streaming. Dynamic policy aspects for other use cases (media processing, uplink streaming etc.) can be studied. Provisioning aspects on M1d interface with respect to network slicing. Study integration/interworking of management API with provisioning aspects of media services with network slicing Aspects related to management of QoS for network slices of media services. How does QoS work with network slicing? Aspects related to realization of media services with multiple network slices, and multi-network slice scenarios. Realization of use cases with network slicing. |

Table 5.12.1.2-2 lists candidate list of Release-17 items that can benefit with further specification on network slicing related aspects.

Table 5.12.1.2-2: Release-17 Items for further Network Slicing related specification

| Work Items | Aspects for study related to network slicing |
| --- | --- |
| 5GMS_EDGE | There is minimal specification in TR 26.803 [46] related to network slicing. The TR can greatly benefit from identifying and specifying network slicing aspects related to edge computing such as below: Use cases: Realization of current edge use cases using network slicing keeping in view the control and management aspects of network slicing architecture as standardized in 3GPP SA2 and SA5. EAS relocation in relation to network slicing impact, architecture, and procedures for supporting EAS relocation with network slicing. |
| EVEX | SA4 has started standardization of a reference architecture for data collection and reporting in TS 26.531 [72]. The TR can benefit from incorporating network slicing related data collection: Study information elements and procedures related to data collection about network slices e.g., from NWDAF as specified in TS 29.520 [70] and TS 28.541 [61]. Slice optimization: Study optimizing network slice parameters for SA4 media services using metrics (analytics) collected using above method. Editor’s Note: Possible study directions include study of how slice related data from NWDAF could be useful, who is the customer of such data, and any required API support to retrieve such data. |
| 5MBUSA | TR 26.802 [73] describes aspects related to multicast. Study can be performed to identify the relationship between 5G multicast and network slicing and answer questions such as below: 5G multicast media service using network slicing: How to realize 5G multicast and broadcast services using network slicing. Hybrid Services: Study network slicing impact on the hybrid services key issue described in TR 26.802 [73]. Investigate whether different network slices can be used with different delivery systems for hybrid services. |
| NOTE: The scope of the EVEX Work Item does not currently include consuming analytics data from the NWDAF. |  |

Editor’s Note: Study to include aspects related to network slice usage e.g., how application/OS/UE can map different application traffic to network slices.

NOTE:	In general, all the items in tables 5.12.1.2-1 and 5.12.1.2-2 are listed here to present areas for further study on aspects related to network slicing. The current scope of each of these work items may not include study of network slicing aspects.

The scope of the study proposed in the above tables is not exhaustive or final. More study topics can be identified in different work areas being discussed in SA4. The study can include how existing and new use cases can benefit with the network slicing specification. However, such a study in SA4 cannot be done in isolation. Multiple study/work items are currently underway in different SA groups. It is recommended that SA4 study consider such studies and work in other groups while specifying media service level network slicing standardization. The following are some of the study/work items in other groups that may be relevant to study in SA4:

-	[3GPP SA2] TR 23.700-40 [71]: Study on enhancement of network slicing; Phase 2.

-	[3GPP SA6] TS 23.434 [68]: Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows.

-	[3GPP SA6] TR 23.700-99 [69]: Study in Network slice capability exposure for application layer enablement (NSCALE).

NOTE:	For maintaining alignment with the specifications in other groups, it is recommended that correspondence with those groups is done using standard 3GPP liaison procedures.

### 5.12.2	Collaboration Scenarios

Editor’s Note: Study collaboration scenarios between the 5G System and Application Provider for each of the key topics.

### 5.12.3	Deployment Architectures

Editor’s Note: Based on the 5GMS Architecture, develop one or more deployment architectures that address the key topics and the collaboration models.

### 5.12.4	Mapping to 5G Media Streaming and High-Level Call Flows

Editor’s Note: Map the key topics to basic functions and develop high-level call flows.

### 5.12.5	Potential open issues

The following potential open issues are identified:

1)	Usage of network slicing identification information as part of service provisioning information (M1)

2)	Applicable stage 2 and stage 3 aspects related to impact to QoS specification during service provisioning because of network slicing

3)	Usage of network slicing identification information during dynamic policy procedures (M5)

4)	Applicable stage 2 and stage 3 aspects related to impact to QoS specification because of network slicing in dynamic policy procedures

5)	Will one AF be sufficient to interact with all network slices? Or, is there a separate AF for each slice?

6)	Provisioning of media services with multiple network slices – Enhancements to M1 interface

7)	QoS Management for network slices – will the QoS be provisioned and managed separately for each slice by the ASP?

8)	Determining the need for AF-to-AF communication to support interoperability if 5GMS AF instances from different vendors are deployed in the same 5GMS System.

NOTE:	There is no impact on SA2 architecture because the concept of a service spanning multiple network slices or PDU Sessions is not in SA2’s scope.

9)	Impacts of network slicing on Rel-16/Rel-17 dynamic policy invocation APIs.

10)	Methods for deploying, supporting, and resolving slice specific AS instances including modification of media description documents (e.g., DASH MPD), DNS etc.

### 5.12.6	Candidate Solutions

#### 5.12.6.1	Network slices and Operation Points provisioned at M1

The 5GMS Application Provider may use the M1 provisioning interface to define a set of network slices that can be used for the media streaming sessions that it offers. This is done when the 5GMS Application Provider would like to request that its media traffic is isolated from other traffic. This may facilitate features such charging and QoS accounting.

It may associate each operation point (e.g. 4K HDR, HD, SD) with a dedicated network slice. Access to each network slice at reference point M4 is restricted to UEs with a valid subscription to that service level. The list or groups of users that are to be authorized to use a certain slice is provided by the 5GMS Application Provider during the provisioning step and can be updated subsequently.

Figure 5.12.6.1-1 below shows a call flow for this solution:

Figure 5.12.6.1-1: Call flow for provisioning network slices and Operation Points

The steps are as follows:

1.	The 5GMS Application Provider provisions the configuration for all upcoming media streaming sessions in the 5GMS AF at reference point M1.

a.	The Application Provider declares a requirement for one or more network slices that correspond to the operation points that it offers for the media streaming service.

b.	The 5GMS AF requests the MnS to create or update a set of network slices based on the provided provisioning information. The MnS for network slice management capability exposure  is currently being studied in SA5 in TR 28.824. The characteristics of a network slice are stored as part of the ServiceProfile as defined in TS 28.541.

c.	The Media Session Handler in the 5GMS Client retrieves Service Access Information from the 5GMS AF. The response includes a mapping between the operation point and the S-NSSAI that should be used by the session.

2.	The 5GMS-Aware Application retrieves the entry point for the media streaming session from the 5GMS Application Provider at reference point M8. The returned entry point contains information about the allowed Operation Points, e.g. in the DASH service descriptor.

3.	The 5GMS-Aware Application selects one of the offered Operation Points.

4.	The application informs the Media Session Handler about the start of a new media streaming session and indicates the selected Operation Point.

5.	The Media Session Handler informs the 5GMS AF about the starting media streaming session and the desired Operation Point.

6.	The 5GMS AF requests the application of the corresponding QoS profile to the media streaming session.

7.	The Media Session Handler may request the establishment of a new PDU session or the  modification of an existing PDU session to use the allowed network slice as indicated by the S-NSSAI. Note that the PDU session is dedicated for the media streaming session traffic. The PDU session establishment and update procedures are defined in clause 8.3 of TS 24.501.

8.	The AMF may request admission control to use the selected slice from the NSACF.

9.	If allowed, the PDU session is established/updated to use the selected S-NSSAI.

10.	Media streaming commences at reference point M4.

To support this high-level procedure:

-	The streaming session uses the eMBB SST slice/service type.

-	The Slice Differntiator is mapped to an operation point of the service that is indicated through the externalReference as defined in clause 7.9.3.1 of TS 26.512 [16].

### 5.12.7	Conclusion

1.	The collaboration scenarios, deployment architectures, and potential open issues concerning network slicing extensions are to be studied further in Release 18.

2.	Impact of network slicing is across multiple work items. Therefore, study for network slicing extensions is to be looked at from the perspective of multiple work items.

# 6	Conclusions

## 6.1	List of Conclusions

Table 6.1-1 points to conclusions and next steps for each of the key issues studied in the present document.

Table 6.1-1: Index of Key Issues, Conclusions, and Next Steps

| Key Issue | Conclusions and Next Steps clause |
| --- | --- |
| Key Issue #1: Content Preparation | 5.2.9 |
| Key Issue #2: Traffic identification | 5.3.7 |
| Key Issue #3: Additional/new transport protocols | 5.4.7 |
| Key Issue #4: Uplink Media Streaming | 5.5.7 |
| Key Issue #5: Background Traffic | 5.6.7 |
| Key Issue #6: Content-Aware Streaming | — |
| Key Issue #7: Network Event usage | 5.8.5 |
| Key Issue #8: Per-application authorization | 5.9.7 |
| Key Issue #9: Support for encrypted and high-value content | — |
| Key Issue #10: TV-grade mass distribution of unicast Live Services | 5.11.7 |
| Key Issue #11: Network Slicing Extensions for 5G Media Streaming | 5.12.7 |

## 6.2	Content preparation

The study of this key issue demonstrates that the use of the Content Preparation Template is not explained adequately in TS 26.501 Release 16. Furthermore, TS 26.512 needs several extensions to make the use of the Content Preparation Template interoperable in the 5GMS architecture.

The following extensions are recommended:

-	Inclusion of content preparation deployment scenarios and associated call flows in Stage 2.

-	Extending the Content Preparation Templates Provisioning API and defining the 5GMS AF/AS requirements needed for the deployment scenarios identified in the study in Stage 3.

## 6.3	Traffic identification

The 5G System offers different solutions for traffic identification and traffic detection. Traffic identification is essential so that the 5G System can detect application flows to which, for instance, an appropriate Policy and Charging Rule (PCC) is applied.

It is recommended to study use-cases with IPsec and the usage of the security parameter index for traffic identification further in a subsequent study.

Existing stage 2 procedures already support the use of different traffic identification mechanisms.

The following extensions are recommended for stage 3:

-	Extensions to the M5 API to support the configuration of ToS values for the identification of traffic associated with a Dynamic Policy.

-	Describe the usage of usage of ToS/DSCP for traffic identification within an informative annex.

## 6.4	Additional/new transport protocols

The latest version of HTTP, HTTP/3, [5] can be used for at least some types of media immediately, but some existing 3GPP Stage 3 specifications are explicitly tied to specific versions of HTTP and need to be updated to allow HTTP/3 as a supported version, in order to allow deployment of this new protocol.

HTTP/3 is defined in the IETF and has been completed in the QUIC working group and approved for publication as an RFC but has not been published yet because of a normative dependency on other documents. The HTTP/3 document is stable (has not changed since February 2021)

When HTTP is used in 5GMS applications, it is often used with application-level protocols such as DASH or HLS that are defined (partially) outside of 3GPP. Some usages in 5GMS will depend on support for HTTP/3 in these application-level protocols.

The following actions are recommended:

-	Update 5GMS stage 3 specifications to allow the use of HTTP/3 at relevant reference points in the 5GMS architecture, to allow early deployment and to identify any unforeseen open issues with using HTTP/3 in a 5G System.

-	At this time, support for HTTP/3 in 5GMS specifications, can be allowed, but not required.

## 6.5	Uplink media streaming

The study of this key issue demonstrates that uplink streaming is severely underspecified in TS 26.501 and TS 26.512 in Release 16, and several gaps are identified.

The following extensions are recommended:

1.	Inclusion of collaboration scenarios and associated call flows in Stage 2.

2.	Specification of egest protocol(s), Content Publishing Configuration APIs and a corresponding Content Publishing Configuration resource, and the UE’s uplink streaming entry point in Stage 3.

## 6.6	Background traffic

Background Data Transfer (BDT) offers Mobile Network Operators (MNOs) and application service providers a tool to deliver content opportunistically at beneficial costs during low traffic time windows. The usage of BDT for media streaming services and potential enhancements to the 5GMS procedures and APIs have been studied and documented in the present document.

Existing stage 2 procedures already support the use of BDT. It is recommended to define the necessary parameter extensions to the M1, M5, and M6 reference points to provide access to BDT. Any normative work has to take into account the available BDT procedures developed by other 3GPP groups.

## 6.7	Content-aware streaming

No conclusion has yet been reached for this key issue.

Initial considerations are provided in clause 5.7. It is recommended to study it further at an appropriate time.

## 6.8	Network Event usage

The 5GMS AF performs several critical support operations for media streaming sessions and collects information about the progress and status of media streaming sessions. This information can be of interest to the 5GMS Application Provider or to other Network Functions in the 5G Sytem.

The following extensions (Stage 2 and Stage 3) are recommended:

-	Identify and define the media-related data to be exposed by the 5GMS AF towards the 5G System and other event consumer entities. Work with other 3GPP groups for carriage in existing AF events or the definition of new AF Events.

-	Enhance the 5GMS AF data collection to support direct and indirect collection of UE data pertaining to media sessions.

-	Devise mechanisms to control the access to the collected media session data.

-	Define a generic architecture within which media-specific solutions for the configuration and subsequent operation of data collection and data reporting (via event exposure) by the AF can be specified.

## 6.9	Per-application-authorization

The 5G Media Streaming Architecture enables a 5GMS-Aware Application to access certain network features, in alignment with the 5GMS Application Provider. The Key Issue explores the usage of OAuth 2.0 for per-application authorization of different 5G System features, for example to prevent misuse. The Key Issue specifically addresses use-cases, when a UE hosts multiple 5GMS-Aware Applications from different 5GMS Application Providers.

It is recommended to specify the usage of OAuth 2.0 (according to the SA3 guidelines) within a normative work item.

## 6.10	Support for encrypted and high-value content

No conclusion has yet been reached for this key issue.

Initial considerations are provided in clause 5.10. It is recommended to study it further at an appropriate time.

## 6.11	TV-grade mass distribution of unicast Live Services

Live TV services of different scale (professional, user-generated, session-based, etc.) are increasingly distributed over broadband and mobile networks, including 5G Networks. Live TV services are characterized by at least the following aspects: (1) scalability (in terms of concurrent users), (2) consistent quality, (3) high bandwidth requirements, (4) target latency constraints, and advanced TV Experiences.

To address these type services a consistent support in the 5G Media Streaming Architecture, the protocols and codecs is needed. It is identified that the combination of low-latency CMAF formats, chunked transfer from content provider to the device, as well as consistent signaling and support of service quality are key aspects to the work.

Based on the discussion and conclusions in clause 5.11.7, it is recommended to support and optimize the deployment of unicast live TV services in 5G Systems. For this purpose, the following follow-up aspects are recommended to be addressed:

1.	Integrate into TS 26.501 (Stage 2)

a)	At least one call flow into that documents provisioning, ingest, distribution, presentation and monitoring aspects of low-latency live streaming services using CMAF Chunks.

b)	Updates to reference points to support provisioning, ingest, distribution, presentation and monitoring aspects of low-latency live services using CMAF Chunks.

c)	Typical configurable service parameters and operation points in terms of bit rates, latencies, Audience Drift Gaps, etc.

2.	Address updates to relevant stage-3 specifications (e.g., TS 26.511, TS 26.512, TS 26.247) to add consistent support of low-latency live streaming services, including:

a)	Define capability mechanisms in order to identify the support of low-latency modes in 5GMS networks and clients.

b)	Provisioning to support operation points and policy templates for low-latency live streaming.

c)	Create necessary extensions to support DASH and HLS chunked CMAF low-latency modes in an end-to-end workflow.

d)	Provide necessary protocols to scalably support time synchronization across 5GMS Applications, AS and 5GMS Clients (at appropriate precision).

e)	Extend QoE metrics schemes and metrics reporting functionality to address monitoring of Operation Point metrics for potential operational improvements.

f)	Provide extensions to formats and manifests support advanced TV experiences.

g)	Informative guidelines on using different Operation Points for low-latency live streaming.

3.	Study even lower-latency streaming technologies based on the use cases and considerations of the DASH-IF webRTC streaming report [94].

## 6.12	Network Slicing Extensions for 5G Media Streaming

As described in TS 28.530, network slicing is a paradigm where logical networks/partitions are created, with appropriate isolation, resources and optimized topology to serve a purpose or service category (e.g. use case/traffic category, or for MNO-internal reasons) or customers (logical system created "on demand"). Media services can be delivered to end users using such different network slices. As documented in clause 5.12 of the present document, aspects related to network slicing are not adequately addressed in TS 26.501 and 26.512. The following are the conclusions so far in the study:

1.	The collaboration scenarios, deployment architectures, and potential open issues concerning network slicing extensions are to be studied further in Release 18.

2.	The impact of network slicing is across multiple work items. Therefore, study for network slicing extensions is to be looked at from the perspective of multiple work items.

# 7	Recommendations

5G Media Streaming provides significant opportunities to integrate operator and third-party media streaming services into 5G Systems. The report provides at set of considered extensions to 5G Media Streaming as defined in TS 26.501, as well as the format and protocol specifications in TS 26.511 and TS 26.512, respectively. Advances in 5G System technologies, external enhancement and developments in other SDOs such as IETF, DASH-IF or MPEG, as well as initial experiences from deployments have led to a set of conclusions in clause 6.

Based on the details in the report, the following next steps are proposed.

1.	Initiate stage 2 and stage 3 work on Network Event usage based on the conclusions in clause 6.8. Note that this is already addressed in TS 26.531 [94] and TS 26.532 [95], respectively.

2.	Provide relevant extensions to the 5G Media Streaming architecture based on the conclusions in clause 6. Candidates for these extensions are:

a)	Content preparation deployment scenarios and associated call flows in Stage 2 according to clause 6.2

b)	Inclusion of collaboration scenarios and associated call flows in Stage 2 for uplink media streaming according to clause 6.5

c)	Inclusion and extensions of procedures and call flows for end-to-end low latency live streaming based on the conclusions in clause 6.11.

3.	Provide relevant extensions to 5G Media Streaming protocols and formats based on the conclusions in clause 6. Candidates for these extensions are:

a)	Stage-3 follow-up work from 5G Media Streaming architecture extensions referred to above based on conclusions in clauses 6.2, 6.5, or 6.11.

b)	Extensions to 5GMS protocols to support traffic identification based on the conclusions in clause 6.3

c)	Addition of HTTP/3 to the 5GMS protocols as an optional alternative based on the conclusions in clause 6.4.

d)	Addition of necessary parameter extensions to the M1, M5, and M6 reference points to provide access to Background Data Transfer based on the conclusions in clause 6.6.

e)	Specification of the usage of OAuth 2.0 (according to the SA3 guidelines) for 5GMS protocols based on the conclusions in clause 6.9.

4.	Continue the study of additional extensions to 5G Media Streaming. Potential candidate topics based on this Technical Report are:

a)	Content-aware streaming based on the initial considerations in clause 5.7.

b)	Study even lower-latency streaming technologies based on the use cases and considerations of the DASH-IF WebRTC streaming report [94].

c)	Distribution of encrypted and high-value content based on the considerations in clause 5.10.

d)	Network slicing extensions for 5G media streaming based on the conclusions in clause 6.12.

All work topics will benefit from continuously checking relevance and support across 3GPP members. In addition, close coordination with other groups in 3GPP on 5G System and radio-related matters, edge computing, applications, operational management and security as well, in communication with experts in MPEG, DASH-IF, CTA WAVE on DASH, HLS and CMAF, as well as with IETF on new protocols.

Annex A:
Media Streaming Protocols

# A.1	Status and usage of Web protocols

The site HTTPArchive.org [x1] offers some insights into the uptake of different HTTP protocol versions by publicly accessible websites. The Report “State of the Web” contains statistics about the number of TCP connections per page and the number of HTTP/2 requests over a time period. The site crawls millions of URLs every month. The URLs are taken from the Chrome User Experience Report.

Currently, around 70% of websites support HTTP/2. Unfortunately, the site does not show statistics for video usage.

The site quic.netray.io [x2] offers some insights into the HTTP/3 (QUIC) take-up.

## A.1.1	M4d protocol usage

It is anticipated that MPEG-DASH would be used by many Application Providers on the M4d Interface if 5GMS services become widely deployed. MPEG-DASH defines the manifest format and also the media segment format. MPEG-DASH allows several different ways to use the underlying HTTP transport, depending on the DASH Profile.

For traffic identification, the identification of the transport protocol (TCP or UDP) used on interface M4d is essential, since the transport protocol needs to be described in the Service Data Flow Template. HTTP/1.1 and HTTP/2 both use TCP transport. HTTP/3 uses a UDP-based QUIC transport. Furthermore, HTTP/1.1. often leverages multiple TCP connections simultaneously, while HTTP/2 and HTTP/3 allow more efficient reuse of the transport through the technique of non-blocking request multiplexing on a single transport connection.

## A.1.2	Results of HTTP protocol version usage study

Editor’s Note: It is currently unclear how to document the results of the transport connection usage study. It is clear, this this represents only a small snapshot on how the different HTTP versions are used and currently only focused on browser based clients.

Within a small study, the transport protocol usage of three major video-on-demand providers were studied, namely YouTube, Netflix and Amazon. The study leveraged browser-based DASH players, using the popular web browsers Google Chrome (version 87.0.4280.141, 64-bit running on Win 10 Pro Version 2004 b 19041.746) and Mozilla Firefox (version 84.0.2, 64-bit running on Win 10 Pro Version 2004 b 19041.746). The intention was to get more insights into HTTP usage.

a)	Accessing YouTube with Chrome, we found that YouTube in a Chrome Browser uses MPEG-DASH with HTTP/3 transport. Several YouTube clips were selected, and HTTP/3 was consistently used for retrieving both media segments and other content. Detailed investigations showed that only a single HTTP/3 connection was established to the server.

b)	Accessing Amazon Prime with Chrome, we found that Amazon Prime uses MPEG-DASH. For some movies, HTTP/2 is used for all content (including media segments). Some other movies used HTTP/1.1 for media segments and HTTP/2 for non-media segments. It is not clear on which basis the application protocol is selected.

c)	Accessing Netflix with Firefox, we found that Netflix uses MPEG-DASH with HTTP/1.1. Some objects, such as images, are fetched using HTTP/2.

d)	Accessing YouTube with Firefox, we found that YouTube uses MPEG-DASH with HTTP/1.1. Non-video transactions use HTTP/2.

Annex X (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-02 | SA4#112-e | S4-210136 |  |  |  | Initial version | 0.0.1 |
| 2021-02 | SA4#112-e | S4-210305 |  |  |  | S4-210054: Key Topic Content Aware Streaming S4-210056: Key Topic Per-application-authorization S4-210298: Key Topic Additional / New transport protocols S4-210302: Key Topic Support for encrypted and high-value content S4-210303: Key Topic Scalable distribution of unicast Live Services | 0.1.0 |
| 2021-03 | Post SA4#112-e CCs | S4-210518 |  |  |  | S4aI201129: Key Topic Uplink media streaming S4aI201126: Key Topic Background traffic S4aI201134: Key Topic Content Preparation (online edited) S4aI201154: [FS_5GMS-EXT] Updated text for Content Preparation (only agreed parts from Clause 5.2.4) S4aI201158: Updates on Key Topic: Network Event usage | 0.1.1 |
| 2021-04 | SA4#113-e | S4-210677 |  |  |  | S4-210637: [FS_5GMS-EXT] Collaboration scenario proposal for KI “per-application-authorization” S4-210636: [FS_5GMS-EXT] Key Topic Additional / New transport protocols S4-210640: [FS_5GMS-EXT] Updated text for Content Preparation | 0.2.0 |
| 2021-04 | Post SA4#113-e CCs | S4-210725 |  |  |  | S4aI201160: [FS_5GMS-EXT] Updated text for uplink streaming S4aI201161: Editorial update to Content Preparation topic S4aI201162: Key Topic Traffic Identification S4aI201166: [FS_5GMS-EXT] Updated text for Content Preparation S4aI211193: Clarification of Traffic Identification description | 0.2.1 |
| 2021-05 | SA4#114-e | S4-210942 |  |  |  | S4-210771: Update on relevant architecture components for traffic identification S4-210912: [FS_5GMS-EXT] New transport protocols/Corrections and Improvements S4-210911: [FS_5GMS-EXT] Key Topic Scalable distribution of unicast Live Services S4-210940: [FS_5GMS-EXT] Candidate solution for Content Preparation format S4-210775: Updated text on uplink streaming S4-210943: Clarification of Traffic Identification description and addition of identified open issues S4-210917: [FS_5GMS-EXT] Updated text for uplink streaming S4-210918: [FS_5GMS-EXT] Updated text for uplink streaming: additional gap analysis | 0.3.0 |
| 2021-06 | SA4#114-e | S4-210960 |  |  |  | S4-210964: Interfaces and Formats for AF Data Collection and Event Exposure | 0.4.0 |
| 2021-06 | Post SA4#114-e |  |  |  |  | Editorial Corrections Correction of S4-210918 implementation Change of 5G logo. | 0.4.1 |
| 2021-08 | SA4#115-e | S4-211240 |  |  |  | S4aI211196: [FS_5GMS_EXT] Content preparation gap analysis: address translation S4-211238: [FS_5GMS-EXT] Content Preparation: Conclusion and recommendations S4-211272: Potential Solutions for Background Data Transfer S4-211239: [FS_5GMS-EXT] Uplink Streaming: Metrics and contribution Reporting S4-211273: [FS_5GMS-EXT] Uplink streaming: Conclusion S4-211274: [FS_5GMS-EXT] New Transport Protocols - Conclusions and Recommendations S4-211275: [FS_5GMS_EXT] Proposal of Candidate Solutions for ToS based traffic detection S4-211276: Network Slicing Extensions for 5G Media Streaming | 0.5.0 |
| 2021-11 | SA4#116e | S4-211599 |  |  |  | S4-211396: [FS_5GMS-EXT] Correct omitted material in 26804-050 merge of S4-211274 S4-211603: Potential Open Issues for Network Slicing Enhancements S4-211653: Potential solution for network slicing in 5GMS S4-211604: [FS_5GMS-EXT] HTTP/3 Deployment Architectures S4-211655: [FS_5GMS_EXT] Corrections and additions for Key Topic Traffic Identification S4-211654: [FS_5GMS-EXT] HTTP/3 collaboration for uplink media streaming S4-211667: [FS_5GMS-EXT] HTTP/3 Candidate Solution - 5GMS OperationMetrics reporting using QLOG events | 0.6.0 |
| 2021-12 | SA#94-e | SP-211340 |  |  |  | Presentation to the plenary for information | 1.0.0 |
| 2022-02 | SA4#117e | S4-220248 |  |  |  | S4-220249: [FS_5GMS-EXT] Updating existing specifications to allow HTTP/3 S4-220245r01: [FS_5GMS_EXT] TV-grade mass distribution of unicast Live Services S4-220247: Conclusion for the BDT key issue S4-220147: [FS_5GMS_EXT] Corrections and Conclusions for Traffic Identification KI S4-220148: [FS_5GMS_EXT]: Per App authorization S4-220172: Additional Potential Issues for Network Slicing Extensions S4-220250: [FS_5GMS_EXT]: Conclusions | 1.1.0 |
| 2022-03 | SA#95-e | SP-220253 |  |  |  | Presentation of Specification to TSG: for approval | 2.0.0 |
| 2022-03 | SA#95-e |  |  |  |  | Under change control | 17.0.0 |