| 3GPP TR 26.830 V0.4.0 (2025-4) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on the enhancement for Immersive Real-Time communication for WebRTC; Phase 2; (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	7

3.3	Abbreviations	7

4	Architectural Assumptions and Requirements	7

4.1	General	7

5	Key Issues	7

5.1	General	7

5.2	Key Issue#1: Media Profiles and Codecs for RTC	7

5.2.1	General	7

5.2.2	Minimum requirements for RTC	8

5.3	Key Issue#2: RTC Signalling and Metadata	8

5.3.1	Description	8

5.4	Key Issue#3: Tethering link delay reporting	8

5.4.1	EVEX approach	8

5.4.2	QoE reporting approach	9

6	Solutions	9

6.1	General	9

6.2	Solution#1: Media Profiles and Codecs for RTC	9

6.2.1	Solution description	9

6.2.2	Proposed Media and Codec Profile for RTC	9

7	Overall Analysis	10

7.1	General	10

8	Conclusions	11

8.1	General	11

Annex A: Change history	11

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

Introduction

This clause is optional. If it exists, it shall be the second unnumbered clause.

1	Scope

The present document identifies possible enhancements of specifications for WebRTC-based real time media communication, to better support of immersive Real-Time Communication (RTC) services. Especially, the following key topics are addressed.

-	Media capabilities, profiles and codecs for RTC

-	Signalling and metadata to support immersive media capabilities

-	Support of tethered cases in RTC system

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 26.113: "Enabler for Immersive Real-time Communication".

[3]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

[4]	3GPP TS 26.119: "Media Capabilities for Augmented Reality".

[5]	3GPP TS 26.531: "Data Collection and Reporting; General Description and Architecture".

[6]	3GPP TR 26.806: "Study on Tethering AR Glasses – Architectures, QoS and Media Aspects".

[7]	3GPP TR 26.812: "QoE Metrics for AR/MR Services".

[8]	3GPP TS 28.405: "Telecommunication management; Quality of Experience (QoE) measurement collection; Control and configuration".

[9]	3GPP TS 26.506: "5G Real-time Media Communication Architecture (Stage 2)".

[10]	IETF RFC 7742 (2016): "WebRTC Video Processing and Codec Requirements".

[11]	IETF RFC 7874 (2016): "WebRTC Audio Codec and Processing Requirements".

[12]	3GPP TS 26.119: "Device Media Capabilities for Augmented Reality Services".

[13]	3GPP TS 26.565: "Split Rendering Media Service Enabler".

[14]	3GPP TS 26.264: "IMS-based AR Real-Time Communication".

[15]	3GPP TR 26.930: "Study on the enhancement for Immersive Real-Time communication for WebRTC".

[16]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

3	Definitions of terms, symbols and abbreviations

This clause and its three (sub) clauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

4	Architectural Assumptions and Requirements

4.1	General

This technical report studies key issues and solutions for enhancements of the RTC specification based on the specifications for stage 2 (i.e., 3GPP TS 26.506 [9]) and stage 3 (i.e., 3GPP TS 26.113 [2]). Therefore, the solutions are required to comply with the RTC architectural requirements.

3GPP TS 26.506 [9] defines Generalized Media Delivery architecture, due to the similarity of the 5GMS architecture to the RTC architecture. Therefore, if a feature common with 5GMS is studied, the Generalized Media Delivery architecture is also considered as the base architecture.

5	Key Issues

5.1	General

Editor's note:	This clause will list the key issues of this study.

5.2	Key Issue#1: Media Profiles and Codecs for RTC

5.2.1	General

In the Rel-18 work, there was a discussion of the media capabilities, profiles, and codecs for RTC endpoints, but no specific codecs nor media capabilities have been derived. Therefore, this key issue addresses the specification of codecs and media capabilities for RTC endpoints.

5.2.2	Minimum requirements for RTC

While 3GPP TS 26.113 [2] mainly specifies the protocols and APIs for RTC (Real-Time media Communication), it also addresses the minimum requirements of media profiles for minimum service interoperability as follows;

A terminal implementing the protocols and APIs defined in 3GPP TS 26.113 [2] should implement:

-	The UE codec requirements for speech as specified in 3GPP TS 26.114 [3], if speech/audio is supported.

-	The UE codec requirements for video as specified in 3GPP TS 26.114 [3], if video is supported.

TS 26.114 provides the following list of codecs to be supported in MTSI clients in terminals;

-	Speech codecs

-	AMR speech codec (mandatory for offering speech communication)

-	AMR-WB codec (mandatory for offering wideband speech communication)

-	EVS codec (mandatory for offering super-wideband or fullband speech communication)

-	IVAS codec (mandatory for offering immersive audio communication)

-	Video codecs

-	H.264 (AVC) Constrained Baseline Profile (CBP) Level 1.2 (mandatory for offering video communication)

-	H.265 (HEVC) Main Profile, Main Tier, Level 3.1 (mandatory for offering video communication)

-	H.264 (AVC) Constrained High Profile (CHP) Level 4.0 (recommended for offering video communication)

-	H.265 (HEVC) Main Profile, Main Tier, Level 4.0 (recommended for offering video communication)

All the codecs above were identified for MTSI (Multimedia Telephony Service for IMS) services, they are assumed to have the capabilities of real-time encoding/decoding. Therefore, all of them may be considered as the candidate media codecs and profiles for RTC services as well.

5.3	Key Issue#2: RTC Signalling and Metadata

5.3.1	Description

In the Rel-18 work, it was specified in 3GPP TS 26.113 [2] that the media and associated metadata either to RTC AS at reference point RTC-4m or to an RTC Access Function in another RTC Client at reference point RTC-12. In addition, the signalling information is exchanged between the RTC Access Function and the WebRTC Signalling Function of an RTC AS at reference point RTC-4s.

During the development of 3GPP TS 26.113 [2], it was raised some essential metadata (e.g., depth sensors) and the signalling context which are required to support the immersive media and to configure the WebRTC connection. In addition, 3GPP TS 26.119 [4] provides the device type and media capabilities identifiers specifically for UEs with immersive media capabilities and available metadata formats for timed metadata of an immersive session, such as pose, action, or visualization space format. However, they should be further specified how to incorporate into RTC architecture.

In this Key Issue, it should be identified how to make them usable in RTC architecture. Especially how to deliver/exchange such information on the RTC architecture.

5.4	Key Issue#3: Support of tethered cases in RTC system

5.4.1	Description

The support for tethered cases in the RTC architecture was studied and documented in 3GPP TR 26.930 [15]. Some work remains to be done:

1.	Identification of the supported scenario for tethered cases. (e.g., "Native WebRTC App/Web App" on the tethered device and "Native WebRTC App/Web App" on the UE.)

2.	Media processing capabilities of the tethered devices and the impact on the RTC architecture.

3.	Evaluate the SA2 solution in 3GPP TS 23.501 [16] and XRM_Ph2 in Release-19 on E2E QoS when there are non-3GPP networks also involved for the use cases considered under SA4 and identify potential gaps and coordinate with SA2 if needed.

6	Solutions

6.1	General

This clause describes the solutions for key issues in clause 5. Each solution corresponds to a key issue with the same number (e.g., Solution #1 corresponds to Key Issue #1).

6.2	Solution#1: Media Profiles and Codecs for RTC

6.2.1	Solution description

This solution addresses Key Issue #1.

This solution studies the media and codec profile for RTC endpoints, which is not specified in RTC specifications during release-18.

6.2.2	Proposed Media and Codec Profile for RTC

This solution proposes the following guidance as the 3GPP profile of the media capability and codec for RTC endpoints in the 3GPP technical specification for RTC media profile.

-	RTC endpoints comply the codec requirements for MTSI client specified in 3GPP TS 26.114 [3] and the codec requirements for XR phone specified in 3GPP TS 26.119 [4].

-	The support condition of each codec for each media (i.e., mandatory, optional, conditional) in the RTC endpoints is taken over the condition specified in 3GPP TS 26.114 [3] and 3GPP TS 26.119 [4].

Table 6.2.2-1, Table 6.2.2-2 and Table 6.2.2-3 provides the informational summary of the supported media and codecs for MTSI client specified in 3GPP TS 26.114 [3] and for XR phone specified in 3GPP TS 26.119 [4]. The definitions of the notation codes in these tables are as follows:

-	M:	Mandatory to support (specified as "shall" in TS documents.)

-	R:	Recommended to support (specified as "should" in TS documents.)

-	O:	Optional to support

Table 6.2.2-1. Summary of audio/speech codecs for MTSI client and XR phone

| Audio codecs Reference | MTSI client TS 26.114 [3] | XR phone TS 26.119 [4] |
| --- | --- | --- |
| AMR | M | - |
| AMR-WB | M | - |
| EVS | M | Decoding: M Encoding: M |
| IVAS | R (M, if immersive) | Decoding: R Encoding: R |
| AAC-ELDv2 | - | Decoding: M Encoding: R |

Table 6.2.2-2. Summary of video codecs for MTSI client and XR phone

| Video codecs Reference | MTSI client TS 26.114 [3] | XR phone TS 26.119 [4] |
| --- | --- | --- |
| H.264 CBP Level 1.2 | M | - |
| H.265 Level 3.1 | M | - |
| H.264 CHP Level 4.0 | R | - |
| H.265 Level 4.0 | R | - |
| H.264 CHP Level 5.1 | O (M, if ITT4RT) | - |
| H.264 PHP Level 5.1 | - | Decoding: M Encoding: M |
| H.265 Level 5.1 | O (M, if ITT4RT) | Decoding: M Encoding: M |
| H.264 PHP Level 6.1 | - | Decoding: R Encoding: - |
| H.265 Level 6.1 | - | Decoding: R Encoding: - |

Table 6.2.2-3. Summary of scene processing capabilities for XR phone

| Scene processing capabilities Reference | XR phone TS 26.119 [4] |
| --- | --- |
| SD-Rendering-glTF-Core | M |
| SD-Rendering-glTF-ext1 | R |
| SD-Rendering-glTF-ext2 | R |
| SD-Rendering-glTF-interactive | R |

RTC architecture supports Web App running in Web browsers. Web browsers support variety of codecs specified in IETF RFC 7742 [10] and IETF RFC 7874 [11] for WebRTC Applications. A media function (which is one of the RTC endpoints) can accommodate the Web App type of RTC endpoint. However, the support of non-3gpp media codecs is outside the scope of RTC specifications. Therefore, this solution also proposes to introduce the following NOTE in the 3GPP technical specification for RTC media profile 3GPP TS 26.113 [2].

-	NOTE: If the RTC endpoint is a Media Function of an RTC AS, the RTC endpoint can support one or more media codecs supported by web browsers for WebRTC applications in order to accommodate Web App running over a Web browser of the UE. However, the support of non-3gpp media codecs is outside the scope of the present document.

6.3	Solution#2: RTC Signalling and Metadata

6.3.1	Solution description

This solution addresses Key Issue #2.

This solution provides the relevant work outcomes which can be incorporated into RTC-based services.

6.3.2	Candidate Metadata for RTC

3GPP TS 26.506 [9] specified the relevant reference points and interfaces to enable real-time media communication including the immersive media such as AR and XR. It implies RTC application supports the exchange of real-time metadata information for about the XR session as defined in 3GPP TS 26.119 [12]. Clause 12.2 of 3GPP TS 26.119 [12] provides the list and the format of timed metadata of an XR session and they are required to be complied to the relevant technical specifications of 3GPP TS 26.565 [13] and 3GPP TS 26.264 [14].

-	Pose format: It is periodically exchanged between the RTC endpoint in the UE and the other RTC endpoint in RTC AS when split rendering is required. The message format of pose is specified in Table 12.2-1 of 3GPP TS 26.119 [12] and the message type may be "urn:3gpp:split-rendering:v1:pose" if applied 3GPP TS 26.565 [13].

-	Action format: It is exchanged per request between the RTC endpoint in the UE and the other RTC endpoint in RTC AS when split rendering is required. The message format of action is specified in Table 12.3-1 of 3GPP TS 26.119 [12] and the message type may be "urn:3gpp:split-rendering:v1:action" if applied 3GPP TS 26.565 [13].

-	Available visualization space format: It may be exchanged to the remote RTC endpoint in RTC AS when the virtual scene is rendered in the RTC AS (such as split rendering). The message format of availableVisualizationSpace is specified in Table 12.4-1 of 3GPP TS 26.119 [12]. If it is required to be supported by RTC application, the message type for split rendering needs to be specified.

The above metadata may be carried over WebRTC data channel at reference point RTC-4m. The data channel message format for split rendering is specified in clause 8.3.3 of 3GPP TS 26.565 [13].

NOTE:	Device type identifier as specified in clause 12.5 of 3GPP TS 26.119 [12] is used for informing the device type and capability of the XR device in metadata. RTC applications may use the identifier to indicate the device capabilities for XR session, however the device type for RTC application is not limited to XR. Thus, the use of this identifier is for further study.

6.3.3	Candidate signalling for RTC

3GPP TS 26.506 [12] specified the reference point RTC-4s to exchange the signalling information for RTC session between the RTC endpoint in the UE and the WebRTC Signalling Function in RTC AS for the purpose of establishing the RTC sessions and managing the media flows. It also includes the information about the available transport protocol, NAT traversal route, network addresses and is carried over WebSocket connection. It may comply with WebRTC APIs such as RTCPeerConnection.

## 6.4	Solution#3: Tethering link delay reporting

6.4.1	Solution description

This solution addresses Key Issue #3.

### 6.4.2	EVEX approach

Clause 6.3 of 3GPP TR 26.806 [6] gives a solution that reports the tethering link delay as an event in the Event Exposure (EVEX) framework in 3GPP TS 26.531 [5].

### 6.4.3	QoE reporting approach

3GPP TR 26.812 [7] includes the round-trip time (RTT) and one-way delays as QoE metrics. The tethering link delay contributes to the RTT and one-way delays, and the knowledge of the tethering link delay may help optimize the communication system. Thus, it is beneficial to include the tethering link delay as a QoE metric. The current QoE measurement and reporting mechanisms in 3GPP TS 28.405 [8] can be leveraged for this purpose.

7	Overall Analysis

7.1	General

Editor's note:	This clause provides analysis of solutions.

8	Conclusions

8.1	General

Editor's note:	This clause will list the conclusions that have been agreed during the study item activities.

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024.11 | SA4#130 | S4-242088 |  |  |  | Initial draft of the TR. S4-242084, S4-242085 and S4aR240074 are incorporated. | 0.1.0 |
| 2025.2 | SA4#131 | S4-250041 |  |  |  | S4aR250051 and S4aR250052 are incorporated. | 0.2.0 |
| 2025.2 | SA4#131 | S4-250397 |  |  |  | S4-250327 is incorporated. | 0.3.1 |
| 2025.4 | SA4#131b | S4-250670 |  |  |  | S4-250437, S4-250506 and S4-250608 are incorporated. | 0.4.0 |