| 3GPP TS 28.541 V18.3.1 (2023-04) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3 (Release 18 ) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	28

Introduction	29

1	Scope	30

2	References	30

3	Definitions of terms, symbols and abbreviations	33

3.1	Terms	33

3.2	Symbols	33

3.3	Abbreviations	33

4	Information model definitions for NR NRM	34

4.1	Imported and associated information	34

4.1.1	Imported information entities and local labels	34

4.1.2	Associated information entities and local labels	34

4.2	Class diagram	34

4.2.1	Class diagram for gNB and en-gNB	34

4.2.1.1	Relationships	34

4.2.1.2	Inheritance	43

4.3	Class definitions	46

4.3.1	GNBDUFunction	46

4.3.1.1	Definition	46

4.3.1.2	Attributes	46

4.3.1.3	Attribute constraints	47

4.3.1.4	Notifications	47

4.3.2	GNBCUCPFunction	47

4.3.2.1	Definition	47

4.3.2.2	Attributes	47

4.3.2.3	Attribute constraints	48

4.3.2.4	Notifications	48

4.3.3	GNBCUUPFunction	48

4.3.3.1	Definition	48

4.3.3.2	Attributes	49

4.3.3.3	Attribute constraints	49

4.3.3.4	Notifications	49

4.3.4	NRCellCU	49

4.3.4.1	Definition	49

4.3.4.2	Attributes	49

4.3.4.3	Void	50

4.3.4.4	Notifications	50

4.3.5	NRCellDU	50

4.3.5.1	Definition	50

4.3.5.2	Attributes	50

4.3.5.3	Attribute constraints	51

4.3.5.4	Notifications	52

4.3.6	NRSectorCarrier	52

4.3.6.1	Definition	52

4.3.6.2	Attributes	52

4.3.6.3	Attribute constraints	52

4.3.6.4	Notifications	53

4.3.7	BWP	53

4.3.7.1	Definition	53

4.3.7.2	Attributes	53

4.3.7.3	Attribute constraints	53

4.3.7.4	Notifications	53

4.3.8	EP_E1	53

4.3.8.1	Definition	53

4.3.8.2	Attributes	53

4.3.8.3	Attribute constraints	54

4.3.8.4	Notifications	54

4.3.9	EP_XnU	54

4.3.9.1	Definition	54

4.3.9.2	Attributes	54

4.3.9.3	Attribute constraints	54

4.3.9.4	Notifications	54

4.3.10	EP_NgC	54

4.3.10.1	Definition	54

4.3.10.2	Attributes	54

4.3.10.3	Attribute constraints	55

4.3.10.4	Notifications	55

4.3.11	EP_NgU	55

4.3.11.1	Definition	55

4.3.11.2	Attributes	55

4.3.11.3	Attribute constraints	55

4.3.11.4	Notifications	55

4.3.12	EP_F1C	55

4.3.12.1	Definition	55

4.3.12.2	Attributes	55

4.3.12.3	Attribute constraints	56

4.3.12.4	Notifications	56

4.3.13	EP_F1U	56

4.3.13.1	Definition	56

4.3.13.2	Attributes	56

4.3.13.3	Attribute constraints	56

4.3.13.4	Notifications	56

4.3.14	EP_S1U	56

4.3.14.1	Definition	56

4.3.14.2	Attributes	56

4.3.14.3	Attribute constraints	57

4.3.14.4	Notifications	57

4.3.15	EP_X2C	57

4.3.15.1	Definition	57

4.3.15.2	Attributes	57

4.3.15.3	Attribute constraints	57

4.3.15.4	Notifications	57

4.3.16	EP_X2U	57

4.3.16.1	Definition	57

4.3.16.2	Attributes	57

4.3.16.3	Attribute constraints	58

4.3.16.4	Notifications	58

4.3.17	EP_XnC	58

4.3.17.1	Definition	58

4.3.17.2	Attributes	58

4.3.17.3	Attribute constraints	58

4.3.17.4	Notifications	58

4.3.18	ExternalGNBCUCPFunction	58

4.3.18.1	Definition	58

4.3.18.2	Attributes	58

4.3.18.3	Attribute constraints	59

4.3.18.4	Notifications	59

4.3.19	ExternalGNBCUUPFunction	59

4.3.19.1	Definition	59

4.3.19.2	Attributes	59

4.3.19.3	Attribute constraints	59

4.3.19.4	Notifications	59

4.3.20	ExternalGNBDUFunction	59

4.3.20.1	Definition	59

4.3.20.2	Attributes	59

4.3.20.3	Attribute constraints	60

4.3.20.4	Notifications	60

4.3.21	ExternalUPFFunction	60

4.3.21.1	Definition	60

4.3.21.2	Attributes	60

4.3.21.3	Attribute constraints	60

4.3.21.4	Notifications	60

4.3.22	ExternalAMFFunction	60

4.3.22.1	Definition	60

4.3.22.2	Attributes	60

4.3.22.3	Attribute constraints	61

4.3.22.4	Notifications	61

4.3.23	Void	61

4.3.24	ENBFunction <<ProxyClass>>	61

4.3.24.1	Definition	61

4.3.24.2	Attributes	61

4.3.24.3	Attribute constraints	61

4.3.24.4	Notifications	61

4.3.25	GNBCUCPFunction <<ProxyClass>>	61

4.3.25.1	Definition	61

4.3.25.2	Attributes	61

4.3.25.3	Attribute constraints	61

4.3.25.4	Notifications	61

4.3.26	GNBCUUPFunction <<ProxyClass>>	62

4.3.26.1	Definition	62

4.3.26.2	Attributes	62

4.3.26.3	Attribute constraints	62

4.3.26.4	Notifications	62

4.3.27	GNBDUFunction <<ProxyClass>>	62

4.3.27.1	Definition	62

4.3.27.2	Attributes	62

4.3.27.3	Attribute constraints	62

4.3.27.4	Notifications	62

4.3.28	ServingGWFFunction <<ProxyClass>>	62

4.3.28.1	Definition	62

4.3.28.2	Attributes	62

4.3.28.3	Attribute constraints	62

4.3.28.4	Notifications	62

4.3.29	UPFFunction <<ProxyClass>>	63

4.3.29.1	Definition	63

4.3.29.2	Attributes	63

4.3.29.3	Attribute constraints	63

4.3.29.4	Notifications	63

4.3.30	AMFFunction <<ProxyClass>>	63

4.3.30.1	Definition	63

4.3.30.2	Attributes	63

4.3.30.3	Attribute constraints	63

4.3.30.4	Notifications	63

4.3.31	Void	63

4.3.32	NRCellRelation	63

4.3.32.1	Definition	63

4.3.32.2	Attributes	63

4.3.32.3	Attribute constraints	64

4.3.32.4	Notifications	64

4.3.33	NRFreqRelation	64

4.3.33.1	Definition	64

4.3.33.2	Attributes	64

4.3.33.3	Attribute constraints	65

4.3.33.4	Notifications	65

4.3.34	Void	65

4.3.35	ExternalNRCellCU	65

4.3.35.1	Definition	65

4.3.35.2	Attributes	65

4.3.35.3	Attribute constraints	66

4.3.35.4	Notifications	66

4.3.36	RRMPolicyRatio	66

4.3.36.1	Definition	66

4.3.36.2	Attributes	67

4.3.36.3	Attribute constraints	67

4.3.36.4	Notifications	67

4.3.37	S-NSSAI <<dataType>>	67

4.3.37.1	Definition	67

4.3.37.2	Attributes	67

4.3.37.3	Attribute constraints	67

4.3.37.4	Notifications	68

4.3.38	NRFrequency	68

4.3.38.1	Definition	68

4.3.38.2	Attributes	68

4.3.38.3	Attribute constraints	68

4.3.38.4	Notifications	68

4.3.39	CommonBeamformingFunction	68

4.3.39.1	Definition	68

4.3.39.2	Attributes	68

4.3.39.3	Attribute constraints	69

4.3.39.4	Notifications	69

4.3.40	Beam	69

4.3.40.1	Definition	69

4.3.40.2	Attributes	69

4.3.40.3	Attribute constraints	69

4.3.41	PLMNInfo <<dataType>>	70

4.3.41.1	Definition	70

4.3.41.2	Attributes	70

4.3.41.3	Attribute constraints	70

4.3.41.4	Notifications	70

4.3.42	RRMPolicyMember <<dataType>>	70

4.3.42.1	Definition	70

4.3.42.2	Attributes	70

4.3.42.3	Attribute constraints	70

4.3.42.4	Notifications	70

4.3.43	RRMPolicy_	71

4.3.43.1	Definition	71

4.3.43.2	Attributes	71

4.3.43.3	Attribute constraints	71

4.3.43.4	Notifications	71

4.3.44	RRMPolicyManagedEntity <<ProxyClass>>	71

4.3.44.1	Definition	71

4.3.44.2	Attributes	72

4.3.44.3	Attribute constraints	72

4.3.44.4	Notifications	72

4.3.45	GNBCUCPNeighbour <<ProxyClass>>	72

4.3.45.1	Definition	72

4.3.45.2	Attributes	72

4.3.45.3	Attribute constraints	72

4.3.45.4	Notifications	72

4.3.46	GNBCUUPNeighbour <<ProxyClass>>	72

4.3.46.1	Definition	72

4.3.46.2	Attributes	72

4.3.46.3	Attribute constraints	72

4.3.46.4	Notifications	72

4.3.47	MappingSetIDBackhaulAddress  <<dataType>>	73

4.3.47.1	Definition	73

4.3.47.2	Attributes	73

4.3.47.3	Attribute constraints	73

4.3.47.4	Notifications	73

4.3.48	BackhaulAddress  <<dataType>>	73

4.3.48.1	Definition	73

4.3.48.2	Attributes	73

4.3.48.3	Attribute constraints	74

4.3.48.4	Notifications	74

4.3.49	TAI <<dataType>>	74

4.3.49.1	Definition	74

4.3.49.2	Attributes	74

4.3.49.3	Attribute constraints	74

4.3.49.4	Notifications	74

4.3.50	RimRSGlobal	74

4.3.50.1	Definition	74

4.3.50.2	Attributes	74

4.3.50.3	Attribute constraints	74

4.3.50.4	Notifications	75

4.3.51	FrequencyDomainPara <<dataType>>	75

4.3.51.1	Definition	75

4.3.51.2	Attributes	75

4.3.51.3	Attribute constraints	75

4.3.51.4	Notifications	75

4.3.52	SequenceDomainPara  <<dataType>>	75

4.3.52.1	Definition	75

4.3.52.2	Attributes	75

4.3.52.3	Attribute constraints	76

4.3.52.4	Notifications	76

4.3.53	TimeDomainPara  <<dataType>>	76

4.3.53.1	Definition	76

4.3.53.2	Attributes	76

4.3.53.3	Attribute constraints	76

4.3.53.4	Notifications	76

4.3.54	RimRSReportConf <<dataType>>	76

4.3.54.1	Definition	76

4.3.54.2	Attributes	76

4.3.54.3	Attribute constraints	77

4.3.54.4	Notifications	77

4.3.55	RimRSReportInfo <<dataType>>	77

4.3.55.1	Definition	77

4.3.55.2	Attributes	77

4.3.55.3	Attribute constraints	77

4.3.55.4	Notifications	77

4.3.56	RimRSSet	77

4.3.56.1	Definition	77

4.3.56.2	Attributes	78

4.3.56.3	Attribute constraints	78

4.3.56.4	Notifications	78

4.3.57	DANRManagementFunction	78

4.3.57.1	Definition	78

4.3.57.2	Attributes	78

4.3.57.3	Attribute constraints	78

4.3.57.4	Notifications	78

4.3.58	DESManagementFunction	78

4.3.58.1	Definition	78

4.3.58.2	Attributes	79

4.3.58.3	Attribute constraints	79

4.3.58.4	Notification	79

4.3.59	DRACHOptimizationFunction	79

4.3.59.1	Definition	79

4.3.59.2	Attributes	80

4.3.59.3	Attribute constraints	80

4.3.59.4	Notifications	80

4.3.60	DMROFunction	80

4.3.60.1	Definition	80

4.3.60.2	Attributes	80

4.3.60.3	Attribute constraints	80

4.3.60.4	Notifications	80

4.3.61	DPCIConfigurationFunction	81

4.3.61.1	Definition	81

4.3.61.2	Attributes	81

4.3.61.3	Attribute constraints	81

4.3.61.4	Notifications	81

4.3.62	CPCIConfigurationFunction	81

4.3.62.1	Definition	81

4.3.62.2	Attributes	81

4.3.62.3	Attribute constraints	81

4.3.62.4	Notifications	82

4.3.63	CESManagementFunction	82

4.3.63.1	Definition	82

4.3.63.2	Attributes	82

4.3.63.3	Attribute constraints	83

4.3.63.4	Notification	83

4.3.64	AddressWithVlan <<dataType>>	83

4.3.64.1	Definition	83

4.3.64.2	Attributes	83

4.3.64.3	Attribute constraints	83

4.3.64.4	Notifications	83

4.3.65	TceIDMappingInfo  <<dataType>>	84

4.3.65.1	Definition	84

4.3.65.2	Attributes	84

4.3.65.3	Attribute constraints	84

4.3.65.4	Notifications	84

4.3.66	NPNIdentity <<dataType>>	84

4.3.66.1	Definition	84

4.3.66.2	Attributes	84

4.3.66.3	Attribute constraints	84

4.3.66.4	Notifications	84

4.3.67	OperatorDU	85

4.3.67.1	Definition	85

4.3.67.2	Attributes	85

4.3.67.3	Attribute Constraints	85

4.3.67.4	Notifications	85

4.3.68	NROperatorCellDU	85

4.3.68.1	Definition	85

4.3.68.2	Attributes	86

4.3.68.3	Attribute Constraints	86

4.3.68.4	Notifications	86

4.3.69	DLBOFunction	86

4.3.69.1	Definition	86

4.3.69.2	Attributes	86

4.3.69.3	Attribute constraints	86

4.3.69.4	Notifications	86

4.3.70	CCOFunction	87

4.3.70.1	Definition	87

4.3.70.2	Attributes	87

4.3.70.3	Attribute constraints	87

4.3.70.4	Notifications	87

4.3.71	CCOWeakCoverageParameters	87

4.3.71.1	Definition	87

4.3.71.2	Attributes	87

4.3.71.3	Attribute constraints	87

4.3.71.4	Notifications	87

4.3.72	CCOPilotPollutionParameters	87

4.3.72.1	Definition	87

4.3.72.2	Attributes	87

4.3.72.3	Attribute constraints	88

4.3.72.4	Notifications	88

4.3.73	CCOOvershootCoverageParameters	88

4.3.73.1	Definition	88

4.3.73.2	Attributes	88

4.3.73.3	Attribute constraints	88

4.3.73.4	Notifications	88

4.3.74	CCOParameters	88

4.3.74.1	Definition	88

4.3.74.2	Attributes	88

4.3.74.3	Attribute constraints	88

4.3.74.4	Notifications	89

4.3.75	ParameterRange <<dataType>>	89

4.3.75.1	Definition	89

4.3.75.2	Attributes	89

4.3.75.3	Attribute constraints	89

4.3.75.4	Notifications	89

4.3.76	BWPSet	89

4.3.76.1	Definition	89

4.3.76.2	Attributes	89

4.3.76.3	Attribute constraints	89

4.3.76.4	Notifications	89

4.4	Attribute definitions	90

4.4.1	Attribute properties	90

4.5	Common notifications	122

4.5.1	Alarm notifications	122

4.5.2	Configuration notifications	122

4.5.3	Threshold Crossing notifications	122

5	Information Model definitions for 5GC NRM	123

5.1	Imported information entities and local labels	123

5.2	Class diagram	123

5.2.1	Class diagram of 5GC NFs	123

5.2.1.1	Relationships	123

5.2.1.2	Inheritance	132

5.2.2	Class diagram of AMF Region/AMF Set	136

5.2.2.1	Relationships	136

5.2.2.2	Inheritance	136

5.3	Class definitions	137

5.3.1	AMFFunction	137

5.3.1.1	Definition	137

5.3.1.2	Attributes	137

5.3.1.3	Attribute constraints	137

5.3.1.4	Notifications	138

5.3.2	SMFFunction	138

5.3.2.1	Definition	138

5.3.2.2	Attributes	138

5.3.2.3	Attribute constraints	138

5.3.2.4	Notifications	138

5.3.3	UPFFunction	138

5.3.3.1	Definition	138

5.3.3.2	Attributes	139

5.3.3.3	Attribute constraints	139

5.3.3.4	Notifications	139

5.3.4	N3IWFFunction	139

5.3.4.1	Definition	139

5.3.4.2	Attributes	139

5.3.4.3	Attribute constraints	140

5.3.4.4	Notifications	140

5.3.5	PCFFunction	140

5.3.5.1	Definition	140

5.3.5.2	Attributes	140

5.3.5.3	Attribute constraints	140

5.3.5.4	Notifications	140

5.3.6	AUSFFunction	141

5.3.6.1	Definition	141

5.3.6.2	Attributes	141

5.3.6.3	Attribute constraints	141

5.3.6.4	Notifications	141

5.3.7	UDMFunction	141

5.3.7.1	Definition	141

5.3.7.2	Attributes	141

5.3.7.3	Attribute constraints	141

5.3.7.4	Notifications	142

5.3.8	UDRFunction	142

5.3.8.1	Definition	142

5.3.8.2	Attributes	142

5.3.8.3	Attribute constraints	142

5.3.8.4	Notifications	142

5.3.9	UDSFFunction	142

5.3.9.1	Definition	142

5.3.9.2	Attributes	142

5.3.9.3	Attribute constraints	142

5.3.9.4	Notifications	143

5.3.10	NRFFunction	143

5.3.10.1	Definition	143

5.3.10.2	Attributes	143

5.3.10.3	Attribute constraints	143

5.3.10.4	Notifications	143

5.3.11	NSSFFunction	143

5.3.11.1	Definition	143

5.3.11.2	Attributes	143

5.3.11.3	Attribute constraints	144

5.3.11.4	Notifications	144

5.3.12	AFFunction	144

5.3.12.1	Definition	144

5.3.13	DNFunction	144

5.3.13.1	Definition	144

5.3.14	SMSFFunction	144

5.3.14.1	Definition	144

5.3.14.2	Attributes	144

5.3.14.3	Attribute constraints	144

5.3.14.4	Notifications	144

5.3.15	LMFFunction	145

5.3.15.1	Definition	145

5.3.15.2	Attributes	145

5.3.15.3	Attribute constraints	145

5.3.15.4	Notifications	145

5.3.16	NGEIRFunction	145

5.3.16.1	Definition	145

5.3.16.2	Attributes	145

5.3.16.3	Attribute constraints	145

5.3.16.4	Notifications	145

5.3.17	SEPPFunction	146

5.3.17.1	Definition	146

5.3.17.2	Attributes	146

5.3.17.3	Attribute constraints	146

5.3.17.4	Notifications	146

5.3.18	NWDAFFunction	146

5.3.18.1	Definition	146

5.3.18.2	Attributes	146

5.3.18.3	Attribute constraints	147

5.3.18.4	Notifications	147

5.3.19	EP_N2	147

5.3.19.1	Definition	147

5.3.19.2	Attributes	147

5.3.19.3	Attribute constraints	147

5.3.19.4	Notifications	147

5.3.20	EP_N3	147

5.3.20.1	Definition	147

5.3.20.2	Attributes	147

5.3.20.3	Attribute constraints	148

5.3.20.4	Notifications	148

5.3.21	EP_N4	148

5.3.21.1	Definition	148

5.3.21.2	Attributes	148

5.3.21.3	Attribute constraints	148

5.3.21.4	Notifications	148

5.3.22	EP_N5	148

5.3.22.1	Definition	148

5.3.22.2	Attributes	148

5.3.22.3	Attribute constraints	148

5.3.22.4	Notifications	148

5.3.23	EP_N6	149

5.3.23.1	Definition	149

5.3.23.2	Attributes	149

5.3.23.3	Attribute constraints	149

5.3.23.4	Notifications	149

5.3.24	EP_N7	149

5.3.24.1	Definition	149

5.3.24.2	Attributes	149

5.3.24.3	Attribute constraints	149

5.3.24.4	Notifications	149

5.3.25	EP_N8	149

5.3.25.1	Definition	149

5.3.25.2	Attributes	150

5.3.25.3	Attribute constraints	150

5.3.25.4	Notifications	150

5.3.26	EP_N9	150

5.3.26.1	Definition	150

5.3.26.2	Attributes	150

5.3.26.3	Attribute constraints	150

5.3.26.4	Notifications	150

5.3.27	EP_N10	150

5.3.27.1	Definition	150

5.3.27.2	Attributes	150

5.3.27.3	Attribute constraints	151

5.3.27.4	Notifications	151

5.3.28	EP_N11	151

5.3.28.1	Definition	151

5.3.28.2	Attributes	151

5.3.28.3	Attribute constraints	151

5.3.28.4	Notifications	151

5.3.29	EP_N12	151

5.3.29.1	Definition	151

5.3.29.2	Attributes	151

5.3.29.3	Attribute constraints	152

5.3.29.4	Notifications	152

5.3.30	EP_N13	152

5.3.30.1	Definition	152

5.3.30.2	Attributes	152

5.3.30.3	Attribute constraints	152

5.3.30.4	Notifications	152

5.3.31	EP_N14	152

5.3.31.1	Definition	152

5.3.31.2	Attributes	152

5.3.31.3	Attribute constraints	152

5.3.31.4	Notifications	153

5.3.32	EP_N15	153

5.3.32.1	Definition	153

5.3.32.2	Attributes	153

5.3.32.3	Attribute constraints	153

5.3.32.4	Notifications	153

5.3.33	EP_N16	153

5.3.33.1	Definition	153

5.3.33.2	Attributes	153

5.3.33.3	Attribute constraints	153

5.3.33.4	Notifications	153

5.3.34	EP_N17	154

5.3.34.1	Definition	154

5.3.34.2	Attributes	154

5.3.34.3	Attribute constraints	154

5.3.34.4	Notifications	154

5.3.35	EP_N20	154

5.3.35.1	Definition	154

5.3.35.2	Attributes	154

5.3.35.3	Attribute constraints	154

5.3.35.4	Notifications	154

5.3.36	EP_N21	154

5.3.36.1	Definition	154

5.3.36.2	Attributes	155

5.3.36.3	Attribute constraints	155

5.3.36.4	Notifications	155

5.3.37	EP_N22	155

5.3.37.1	Definition	155

5.3.37.2	Attributes	155

5.3.37.3	Attribute constraints	155

5.3.37.4	Notifications	155

5.3.38	EP_N26	155

5.3.38.1	Definition	155

5.3.38.2	Attributes	155

5.3.38.3	Attribute constraints	156

5.3.38.4	Notifications	156

5.3.39	Void	156

5.3.40	Void	156

5.3.41	EP_S5C	156

5.3.41.1	Definition	156

5.3.41.2	Attributes	156

5.3.41.3	Attribute constraints	156

5.3.41.4	Notifications	156

5.3.42	EP_S5U	156

5.3.42.1	Definition	156

5.3.42.2	Attributes	156

5.3.42.3	Attribute constraints	157

5.3.42.4	Notifications	157

5.3.43	EP_Rx	157

5.3.43.1	Definition	157

5.3.43.2	Attributes	157

5.3.43.3	Attribute constraints	157

5.3.43.4	Notifications	157

5.3.44	EP_MAP_SMSC	157

5.3.44.1	Definition	157

5.3.44.2	Attributes	157

5.3.44.3	Attribute constraints	158

5.3.44.4	Notifications	158

5.3.45	EP_NLS	158

5.3.45.1	Definition	158

5.3.45.2	Attributes	158

5.3.45.3	Attribute constraints	158

5.3.45.4	Notifications	158

5.3.46	EP_NLG	158

5.3.46.1	Definition	158

5.3.46.2	Attributes	158

5.3.46.3	Attribute constraints	158

5.3.46.4	Notifications	159

5.3.47	EP_N27	159

5.3.47.1	Definition	159

5.3.47.2	Attributes	159

5.3.47.3	Attribute constraints	159

5.3.47.4	Notifications	159

5.3.48	EP_N31	159

5.3.48.1	Definition	159

5.3.48.2	Attributes	159

5.3.48.3	Attribute constraints	159

5.3.48.4	Notifications	159

5.3.49	ExternalNRFFunction	160

5.3.49.1	Definition	160

5.3.49.2	Attributes	160

5.3.49.3	Attribute constraints	160

5.3.49.4	Notifications	160

5.3.50	ExternalNSSFFunction	160

5.3.50.1	Definition	160

5.3.50.2	Attributes	160

5.3.50.3	Attribute constraints	160

5.3.50.4	Notifications	160

5.3.51	AMFSet	161

5.3.51.1	Definition	161

5.3.51.2	Attributes	161

5.3.51.3	Attribute constraints	161

5.3.51.4	Notifications	161

5.3.52	AMFRegion	161

5.3.52.1	Definition	161

5.3.52.2	Attributes	161

5.3.52.3	Attribute constraints	162

5.3.52.4	Notifications	162

5.3.53	ExternalAMFFunction	162

5.3.53.1	Definition	162

5.3.53.2	Attributes	162

5.3.53.3	Attribute constraints	162

5.3.53.4	Notifications	162

5.3.54	ManagedNFProfile <<dataType>>	162

5.3.54.1	Definition	162

5.3.54.2	Attributes	163

5.3.54.3	Attribute constraints	163

5.3.54.4	Notifications	163

5.3.55	HostAddr <<choice>>	163

5.3.55.1	Definition	163

5.3.56	Void	164

5.3.57	Void	164

5.3.58	AusfInfo <<dataType>>	164

5.3.58.1	Definition	164

5.3.58.2	Attributes	164

5.3.58.3	Attribute constraints	164

5.3.58.4	Notifications	164

5.3.59	Void	164

5.3.60	Void	164

5.3.61	Void	165

5.3.62	EP_N32	165

5.3.62.1	Definition	165

5.3.62.2	Attributes	165

5.3.62.3	Attribute constraints	165

5.3.62.4	Notifications	165

5.3.63	ExternalSEPPFunction	165

5.3.63.1	Definition	165

5.3.63.2	Attributes	165

5.3.63.3	Attribute constraints	165

5.3.63.4	Notifications	166

5.3.64	SEPPFunction <<ProxyClass>>	166

5.3.64.1	Definition	166

5.3.64.2	Attributes	166

5.3.64.3	Attribute constraints	166

5.3.64.4	Notifications	166

5.3.65	NEFFunction	166

5.3.65.1	Definition	166

5.3.65.2	Attributes	166

5.3.65.3	Attribute constraints	167

5.3.65.4	Notifications	167

5.3.66	SCPFunction	167

5.3.67.1	Definition	167

5.3.67.2	Attributes	167

5.3.67.3	Attribute constraints	167

5.3.67.4	Notifications	167

5.3.68	SupportedFunction <<dataType>>	167

5.3.68.1	Definition	167

5.3.68.2	Attributes	167

5.3.68.3	Attribute constraints	167

5.3.68.4	Notifications	168

5.3.69	CommModel <<dataType>>	168

5.3.69.1	Definition	168

5.3.69.2	Attributes	168

5.3.69.3	Attribute constraints	168

5.3.69.4	Notifications	168

5.3.70	QFQoSMonitoringControl	168

5.3.70.1	Definition	168

5.3.70.2	Attributes	168

5.3.70.3	Attribute constraints	169

5.3.70.4	Notifications	169

5.3.71	QFDelayThresholdsType <<dataType>>	169

5.3.71.1	Definition	169

5.3.71.2	Attributes	169

5.3.71.3	Attribute constraints	169

5.3.71.4	Notifications	169

5.3.72	GtpUPathQoSMonitoringControl	169

5.3.72.1	Definition	169

5.3.72.2	Attributes	169

5.3.72.3	Attribute constraints	170

5.3.72.4	Notifications	170

5.3.73	GtpUPathDelayThresholdsType <<dataType>>	170

5.3.73.1	Definition	170

5.3.73.2	Attributes	170

5.3.73.3	Attribute constraints	170

5.3.73.4	Notifications	170

5.3.75	Configurable5QISet	171

5.3.75.1	Definition	171

5.3.75.2	Attributes	171

5.3.75.3	Attribute constraints	171

5.3.75.4	Notifications	171

5.3.76	FiveQICharacteristics	171

5.3.76.1	Definition	171

5.3.76.2	Attributes	171

5.3.76.3	Attribute constraints	171

5.3.76.4	Notifications	171

5.3.77	PacketErrorRate <<dataType>>	172

5.3.77.1	Definition	172

5.3.77.2	Attributes	172

5.3.77.3	Attribute constraints	172

5.3.77.4	Notifications	172

5.3.78	FiveQiDscpMappingSet	172

5.3.78.1	Definition	172

5.3.78.2	Attributes	172

5.3.78.3	Attribute constraints	172

5.3.78.4	Notifications	172

5.3.79	FiveQiDscpMapping <<dataType>>	172

5.3.79.1	Definition	172

5.3.79.2	Attributes	173

5.3.79.3	Attribute constraints	173

5.3.79.4	Notifications	173

5.3.80	PredefinedPccRuleSet	173

5.3.80.1	Definition	173

5.3.80.2	Attributes	173

5.3.80.3	Attribute constraints	173

5.3.80.4	Notifications	173

5.3.81	PccRule <<dataType>>	173

5.3.81.1	Definition	173

5.3.81.2	Attributes	174

5.3.81.3	Attribute constraints	174

5.3.81.4	Notifications	174

5.3.82	FlowInformation <<dataType>>	174

5.3.82.1	Definition	174

5.3.82.2	Attributes	174

5.3.82.3	Attribute constraints	175

5.3.82.4	Notifications	175

5.3.83	EthFlowDescription <<dataType>>	175

5.3.83.1	Definition	175

5.3.83.2	Attributes	175

5.3.83.3	Attribute constraints	175

5.3.83.4	Notifications	175

5.3.84	QoSData <<dataType>>	175

5.3.84.1	Definition	175

5.3.84.2	Attributes	176

5.3.84.3	Attribute constraints	176

5.3.84.4	Notifications	176

5.3.85	ARP <<dataType>>	176

5.3.85.1	Definition	176

5.3.85.2	Attributes	176

5.3.85.3	Attribute constraints	176

5.3.85.4	Notifications	176

5.3.86	TrafficControlData <<dataType>>	177

5.3.86.1	Definition	177

5.3.86.2	Attributes	177

5.3.86.3	Attribute constraints	177

5.3.86.4	Notifications	177

5.3.87	RedirectInformation <<dataType>>	177

5.3.87.1	Definition	177

5.3.87.2	Attributes	177

5.3.87.3	Attribute constraints	177

5.3.87.4	Notifications	178

5.3.88	RouteToLocation <<dataType>>	178

5.3.88.1	Definition	178

5.3.88.2	Attributes	178

5.3.88.3	Attribute constraints	178

5.3.88.4	Notifications	178

5.3.89	RouteInformation <<dataType>>	178

5.3.89.1	Definition	178

5.3.89.2	Attributes	178

5.3.89.3	Attribute constraints	178

5.3.89.4	Notifications	179

5.3.90	UpPathChgEvent <<dataType>>	179

5.3.90.1	Definition	179

5.3.90.2	Attributes	179

5.3.90.3	Attribute constraints	179

5.3.90.4	Notifications	179

5.3.91	SteeringMode <<dataType>>	179

5.3.91.1	Definition	179

5.3.91.2	Attributes	179

5.3.91.3	Attribute constraints	180

5.3.91.4	Notifications	180

5.3.92	ConditionData  <<dataType>>	180

5.3.92.1	Definition	180

5.3.92.2	Attributes	180

5.3.92.3	Attribute constraints	180

5.3.92.4	Notifications	180

5.3.93	TscaiInputContainer  <<dataType>>	180

5.3.93.1	Definition	180

5.3.93.2	Attributes	180

5.3.93.3	Attribute constraints	181

5.3.93.4	Notifications	181

5.3.94	Dynamic5QISet	181

5.3.94.1	Definition	181

5.3.94.2	Attributes	181

5.3.94.3	Attribute constraints	181

5.3.94.4	Notifications	181

5.3.95	NetworkSliceInfo <<dataType>>	181

5.3.95.1	Definition	181

5.3.95.2	Attributes	181

5.3.95.3	Attribute constraints	181

5.3.95.4	Notifications	182

5.3.96	NSACFFunction	182

5.3.96.1	Definition	182

5.3.96.2	Attributes	182

5.3.96.3	Attribute constraints	182

5.3.96.4	Notifications	182

5.3.97	SnssaiInfo <<dataType>>	182

5.3.97.1	Definition	182

5.3.97.2	Attributes	182

5.3.97.3	Attribute constraints	182

5.3.97.4	Notifications	182

5.3.98	NsacfInfoSnssai <<dataType>>	183

5.3.98.1	Definition	183

5.3.98.2	Attributes	183

5.3.98.3	Attribute constraints	183

5.3.98.4	Notifications	183

5.3.99	EP_N60	183

5.3.99.1	Definition	183

5.3.99.2	Attributes	183

5.3.99.3	Attribute constraints	183

5.3.99.4	Notifications	184

5.3.100	EP_N33	184

5.3.100.1	Definition	184

5.3.100.3	Attributes	184

5.3.100.3	Attribute constraints	184

5.3.100.4	Notifications	184

5.3.101	DDNMFFunction	184

5.3.101.1	Definition	184

5.3.101.2	Attributes	184

5.3.101.3	Attribute constraints	184

5.3.101.4	Notifications	184

5.3.102	EP_Npc4	185

5.3.102.1	Definition	185

5.3.102.2	Attributes	185

5.3.102.3	Attribute constraints	185

5.3.102.4	Notifications	185

5.3.103	EP_Npc6	185

5.3.103.1	Definition	185

5.3.103.2	Attributes	185

5.3.103.3	Attribute constraints	185

5.3.103.4	Notifications	185

5.3.104	EP_Npc7	186

5.3.104.1	Definition	186

5.3.104.2	Attributes	186

5.3.104.3	Attribute constraints	186

5.3.104.4	Notifications	186

5.3.105	GUAMInfo <<dataType>>	186

5.3.105.1	Definition	186

5.3.105.2	Attributes	186

5.3.105.3	Notifications	186

5.3.106	TaiRange <<dataType>>	186

5.3. 106.1	Definition	186

5.3.106.2	Attributes	186

5.3.106.3	Notifications	186

5.3.107	nRTACRange <<dataType>>	187

5.3.107.1	Definition	187

5.3.107.2	Attributes	187

Either the start and end attributes, or the pattern attribute, shall be present.	187

5.3.107.3	Notifications	187

5.3.108	SCPInfo <<dataType>>	187

5.3.108.1	Definition	187

5.3.108.2	Attributes	187

5.3.108.3	Notifications	187

5.3.109	SCPDomainInfo <<dataType>>	187

5.3.109.1	Definition	187

5.3.109.2	Attributes	188

5.3.109.3	Notifications	188

5.3.110	IpEndPoint <<dataType>>	188

5.3.110.1	Definition	188

5.3.110.2	Attributes	188

5.3.110.3	Notifications	188

5.3.111	IPv4AddressRange <<dataType>>	188

5.3.111.1	Definition	188

5.3.111.2	Attributes	188

5.3.111.3	Notifications	188

5.3.112	IPv6PrefixRange <<dataType>>	189

5.3.112.1	Definition	189

5.3.112.2	Attributes	189

5.3.112.3	Notifications	189

5.3.113	EASDFFunction	189

5.3.113.1	Definition	189

5.3.113.2	Attributes	189

5.3.114	EP_N88x	189

5.3.114.1	Definition	189

5.3.114.2	Attributes	189

5.3.114.3	Attribute constraints	190

5.3.114.4	Notifications	190

5.3.115	SNPNInfo <<dataType>>	190

5.3.115.1	Definition	190

5.3.115.2	Attributes	190

5.3.115.3	Attribute constraints	190

5.3.115.4	Notifications	190

5.3.116	SNPNId <<dataType>>	190

5.3.116.1	Definition	190

5.3.116.2	Attributes	190

5.3.116.3	Notifications	190

5.3.117	EP_Npc8	190

5.3.117.1	Definition	190

5.3.117.2	Attributes	190

5.3.117.3	Attribute constraints	191

5.3.117.4	Notifications	191

5.3.118	DefaultNotificationSubscription <<dataType>>	191

5.3.118.1	Definition	191

5.3.118.2	Attributes	191

5.3.118.3	Notifications	191

5.3.119	EcmConnectionInfo	191

5.3.119.1	Definition	191

5.3.119.2	Attributes	191

5.3.119.3	Attribute constraints	192

5.3.119.4	Notifications	192

5.3.120	5GCNfConnEcmInfo <<dataType>>	192

5.3.120.1	Definition	192

5.3.120.2	Attributes	192

5.3.120.3	Attribute constraints	192

5.3.120.4	Notifications	192

5.3.121	UPFConnInfo <<dataType>>	193

5.3.121.1	Definition	193

5.3.121.2	Attributes	193

5.3.121.3	Attribute constraints	193

5.3.121.4	Notifications	193

5.3.122	SnssaiSmfInfoItem <<dataType>>	193

5.3.122.1	Definition	193

5.3.122.2	Attributes	193

5.3.122.3	Notifications	193

5.3.123	DnnSmfInfoItem <<dataType>>	193

5.3.123.1	Definition	193

5.3.123.2	Attributes	193

5.3.123.3	Notifications	193

5.3.124	IpAddr <<dataType>>	194

5.3.124.1	Definition	194

5.3.124.2	Attributes	194

5.3.124.3	Notifications	194

5.3.125	InterfaceUpfInfoItem <<dataType>>	194

5.3.125.1	Definition	194

5.3.125.2	Attributes	194

5.3.125.3	Notifications	194

5.3.126	AtsssCapability <<dataType>>	194

5.3.126.1	Definition	194

5.3.126.2	Attributes	194

5.3.126.3 Attribute constraints	195

5.3.126.4	Notifications	195

5.3.127	IpInterface <<dataType>>	195

5.3.127.1	Definition	195

5.3.127.2	Attributes	195

5.3.127.3	Notifications	195

5.3.128	SupiRange <<dataType>>	195

5.3.128.1	Definition	195

5.3.128.2	Attributes	195

5.3.128.3	Attribute constraints	195

5.3.128.4	Notifications	195

5.3.129	IdentityRange <<dataType>>	196

5.3.Y.1	Definition	196

5.3.129.2	Attributes	196

5.3.129.3	Attribute constraints	196

5.3.129.4	Notifications	196

5.3.130	ProSeCapability <<dataType>>	196

5.3.130.1	Definition	196

5.3.130.2	Attributes	196

5.3.130.3	Attribute constraints	196

5.3.130.4	Notifications	196

5.3.131	V2xCapability <<dataType>>	196

5.3.131.1	Definition	196

5.3.131.2	Attributes	197

5.3.131.3	Attribute constraints	197

5.3.131.4	Notifications	197

5.3.132	InternalGroupIdRange <<dataType>>	197

5.3.132.1	Definition	197

5.3.132.2	Attributes	197

5.3.132.3	Attribute constraints	197

5.3.132.4	Notifications	197

5.3.133	SuciInfo <<dataType>>	197

5.3.133.1	Definition	197

5.3.133.2	Attributes	197

5.3.133.3	Attribute constraints	197

5.3.133.4	Notifications	198

5.3.134	SharedDataIdRange <<dataType>>	198

5.3.134.1	Definition	198

5.3.134.2	Attributes	198

5.3.134.3	Attribute constraints	198

5.3.134.4	Notifications	198

5.3.135	PfdData <<dataType>>	198

5.3.135.1	Definition	198

5.3.135.2	Attributes	198

5.3.135.3	Attribute constraints	198

5.3.135.4	Notifications	198

5.3.136	AfEventExposureData <<dataType>>	198

5.3.136.1	Definition	198

5.3.136.2	Attributes	199

5.3.136.3	Attribute constraints	199

5.3.136.4	Notifications	199

5.3.137	UnTrustAfInfo <<dataType>>	199

5.3.137.1	Definition	199

5.3.137.2	Attributes	199

5.3.137.3	Attribute constraints	199

5.3.137.4	Notifications	199

5.3.138	SnssaiInfoItem <<dataType>>	199

5.3.138.1	Definition	199

5.3.138.2	Attributes	199

5.3.138.3	Attribute constraints	199

5.3.138.4	Notifications	200

5.3.139	ExtSnssai <<dataType>>	200

5.3.139.1	Definition	200

5.3.139.2	Attributes	200

5.3.139.3	Attribute constraints	200

5.3.139.4	Notifications	200

5.3.140	SnssaiExtension <<dataType>>	200

5.3.140.1	Definition	200

5.3.140.2	Attributes	200

5.3.140.3	Attribute constraints	200

5.3.140.4	Notifications	200

5.3.141	DnnInfoItem <<dataType>>	200

5.3.141.1	Definition	200

5.3.141.2	Attributes	201

5.3.141.3	Attribute constraints	201

5.3.141.4	Notifications	201

5.3.142	SdRange <<dataType>>	201

5.3.142.1	Definition	201

5.3.142.2	Attributes	201

5.3.142.3	Attribute constraints	201

5.3.142.4	Notifications	201

5.3.125	NsacfInfo <<dataType>>	201

5.3.125.1	Definition	201

5.3.125.2	Attributes	201

5.3.125.3	Attribute constraints	201

5.3.125.4	Notifications	201

5.3.126	NsacfCapability <<dataType>>	202

5.3.126.1	Definition	202

5.3.126.2	Attributes	202

5.3.126.3	Attribute constraints	202

5.3.126.4	Notifications	202

5.3.127	NwdafInfo <<dataType>>	202

5.3.127.1	Definition	202

5.3.127.2	Attributes	202

5.3.127.3	Attribute constraints	202

5.3.127.4	Notifications	202

5.3.128	NwdafCapability <<dataType>>	202

5.3.238.1	Definition	202

5.3.128.2	Attributes	203

5.3.128.3	Attribute constraints	203

5.3.128.4	Notifications	203

5.3.129	MlAnalyticsInfo <<dataType>>	203

5.3.129.1	Definition	203

5.3.129.2	Attributes	203

5.3.129.3	Attribute constraints	203

5.3.129.4	Notifications	203

5.3.130	PlmnIdNid <<dataType>>	203

5.3.130.1	Definition	203

5.3.130.2	Attributes	203

5.3.130.3	Attribute constraints	204

5.3.130.4	Notifications	204

5.3.131	SeppInfo <<dataType>>	204

5.3.131.1	Definition	204

5.3.131.2	Attributes	204

5.3.131.3	Attribute constraints	204

5.3.131.4	Notifications	204

5.3.132	UdsfInfo <<dataType>>	204

5.3.132.1	Definition	204

5.3.132.2	Attributes	204

5.3.132.3	Attribute constraints	204

5.3.132.4	Notifications	204

5.4	Attribute definitions	206

5.4.1	Attribute properties	206

5.5	Common notifications	249

5.5.1	Alarm notifications	249

5.5.2	Configuration notifications	250

5.5.3	Threshold Crossing notifications	250

5A	Information model definitions for SBA support of IMS	250

5A.1	Imported information entities and local labels	250

5A.2	Class diagram	250

5A.2.1	Class diagram for SBA support of IMS	250

5A.2.1.1	 Relationships	250

5A.2.1.2	Inheritance	251

5A.3	Class definitions	252

5A.3.1	EP_N5	252

5A.3.1.1	Definition	252

5A.3.1.2	Attributes	252

5A.3.2	EP_N70	252

5A.3.2.1	Definition	252

5A.3.2.2	Attributes	252

5A.3.3	EP_N71	252

5A.3.3.1	Definition	252

5A.3.3.2	Attributes	253

6	Information model definitions for network slice NRM	253

6.1	Imported information entities and local labels	253

6.2	Class diagram	253

6.2.1	Relationships	253

6.2.2	Inheritance	255

6.3	Class definitions	256

6.3.1	NetworkSlice	256

6.3.1.1	Definition	256

6.3.1.2	Attributes	256

6.3.1.3	Attribute constraints	256

6.3.1.4	Notifications	256

6.3.2	NetworkSliceSubnet	256

6.3.2.1	Definition	256

6.3.2.2	Attributes	257

6.3.2.3	Attribute constraints	257

6.3.2.4	Notifications	257

6.3.3	ServiceProfile <<dataType>>	257

6.3.3.1	Definition	257

6.3.3.2	Attributes	258

6.3.3.3	Attribute constraints	258

6.3.3.4	Notifications	258

6.3.4	SliceProfile <<dataType>>	259

6.3.4.1	Definition	259

6.3.4.2	Attributes	259

6.3.4.3	Attribute constraints	259

6.3.4.4	Notifications	259

6.3.5	NsInfo <<dataType>>	259

6.3.5.1	Definition	259

6.3.5.2	Attributes	259

6.3.5.3	Attribute constraints	259

6.3.5.4	Notifications	260

6.3.6	ServAttrCom <<dataType>>	260

6.3.6.1	Definition	260

6.3.6.2	Attributes	260

6.3.6.3	Attribute constraints	260

6.3.6.4	Notifications	260

6.3.7	DelayTolerance<<dataType>>	260

6.3.7.1	Definition	260

6.3.7.2	Attributes	260

6.3.7.3	Attribute constraints	260

6.3.7.4	Notifications	260

6.3.8	DeterminComm <<dataType>>	261

6.3.8.1	Definition	261

6.3.8.2	Attributes	261

6.3.8.3	Attribute constraints	261

6.3.8.4	Notifications	261

6.3.9	XLThpt<<dataType>>	261

6.3.9.1	Definition	261

6.3.9.2	Attributes	261

6.3.9.3	Attribute constraints	261

6.3.9.4	Notifications	262

6.3.10	Void	262

6.3.11	MaxPktSize <<dataType>>	262

6.3.11.1	Definition	262

6.3.11.2	Attributes	262

6.3.11.3	Attribute constraints	262

6.3.11.4	Notifications	262

6.3.12	MaxNumberofPDUSessions <<dataType>>	262

6.3.12.1	Definition	262

6.3.12.2	Attributes	262

6.3.12.3	Attribute constraints	263

6.3.12.4	Notifications	263

6.3.13	Void	263

6.3.14	KPIMonitoring <<dataType>>	263

6.3.14.1	Definition	263

6.3.14.2	Attributes	263

6.3.14.3	Attribute constraints	263

6.3.14.4	Notifications	263

6.3.15	UserMgmtOpen<<dataType>>	263

6.3.15.1	Definition	263

6.3.15.2	Attributes	263

6.3.15.3	Attribute constraints	263

6.3.15.4	Notifications	264

6.3.16	V2XCommMode<<dataType>>	264

6.3.16.1	Definition	264

6.3.16.2	Attributes	264

6.3.16.3	Attribute constraints	264

6.3.16.4	Notifications	264

6.3.17	TermDensity<<dataType>>	264

6.3.17.1	Definition	264

6.3.17.2	Attributes	264

6.3.17.3	Attribute constraints	264

6.3.17.4	Notifications	264

6.3.18	EP_Transport	264

6.3.18.1	Definition	264

6.3.18.2	Attributes	265

6.3.18.3	Attribute constraints	265

6.3.18.4	Notifications	265

6.3.19	EP_Application <<ProxyClass>>	265

6.3.19.1	Definition	265

6.3.19.2	Attributes	265

6.3.19.3	Attribute constraints	265

6.3.19.4	Notifications	265

6.3.20	NBIoT <<dataType>>	265

6.3.20.1	Definition	265

6.3.20.2	Attributes	266

6.3.20.3	Attribute constraints	266

6.3.20.4	Notifications	266

6.3.21	Void	266

6.3.22	Void	266

6.3.23	CNSliceSubnetProfile<<dataType>>	266

6.3.23.1	Definition	266

6.3.23.2	Attributes	266

6.3.23.3	Attribute constraints	266

6.3.23.4	Notifications	267

6.3.24	RANSliceSubnetProfile<<dataType>>	267

6.3.24.1	Definition	267

6.3.24.2	Attributes	267

6.3.24.3	Attribute constraints	267

6.3.24.4	Notifications	267

6.3.25	TopSliceSubnetProfile<<dataType>>	267

6.3.25.1	Definition	267

6.3.25.2	Attributes	268

6.3.25.3	Attribute constraints	268

6.3.25.4	Notifications	268

6.3.26	Positioning <<dataType>>	268

6.3.26.1	Definition	268

6.3.26.2	Attributes	268

6.3.26.3	Attribute constraints	269

6.3.26.4	Notifications	269

6.3.27	Synchronicity <<dataType>>	269

6.3.27.1	Definition	269

6.3.27.2	Attributes	269

6.3.27.3	Attribute constraints	269

6.3.27.4	Notifications	269

6.3.28	PositioningRANSubnet <<dataType>>	269

6.3.28.1	Definition	269

6.3.28.2	Attributes	269

6.3.28.3	Attribute constraints	269

6.3.28.4	Notifications	269

6.3.29	SynchronicityRANSubnet <<dataType>>	270

6.3.29.1	Definition	270

6.3.29.2	Attributes	270

6.3.29.3	Attribute constraints	270

6.3.29.4	Notifications	270

6.3.30	EnergyEfficiency <<dataType>>	270

6.3.30.1	Definition	270

6.3.30.2	Attributes	270

6.3.30.3	Attribute constraints	270

6.3.30.4	Notifications	270

6.3.31	RadioSpectrum <<dataType>>	270

6.3.31.1	Definition	270

6.3.31.2	Attributes	271

6.3.31.3	Attribute constraints	271

6.3.31.4	Notifications	271

6.3.32	N6Protection <<dataType>>	271

6.3.32.1	Definition	271

6.3.32.2	Attributes	271

6.3.32.3	Attribute constraints	271

6.3.32.4	Notifications	271

6.3.33	SecFunc <<dataType>>	271

6.3.33.1	Definition	271

6.3.33.2	Attributes	272

6.3.33.3	Attribute constraints	272

6.3.33.4	Notifications	272

6.3.34	NSSAASupport <<dataType>>	272

6.3.34.1	Definition	272

6.3.34.2	Attributes	272

6.3.34.3	Attribute constraints	272

6.3.34.4	Notifications	272

6.3.35	LogicalInterfaceInfo <<dataType>>	272

6.3.35.1	Definition	272

6.3.35.2	Attributes	272

6.3.35.3	Attribute constraints	272

6.3.35.4	Notifications	273

6.3.36	NetworkSliceSubnetProviderCapabilities	273

6.3.36.1	Definition	273

6.3.36.2	Attributes	273

6.3.36.3	Attribute constraints	273

6.3.36.4	Notifications	273

6.3.37	FeasibilityCheckAndReservationJob	273

6.3.37.1	Definition	273

6.3.37.2	Attributes	274

6.3.37.3	Attribute constraints	275

6.3.37.4	Notifications	275

6.3.38	MaxNumberofUEs <<dataType>>	275

6.3.38.1	Definition	275

6.3.38.2	Attributes	275

6.3.38.3	Attribute constraints	275

6.3.38.4	Notifications	275

6.4	Attribute definition	276

6.4.1	Attribute properties	276

6.5	Common notifications	292

6.5.1	Alarm notifications	292

6.5.2	Configuration notifications	292

6.5.3	Threshold Crossing notifications	292

7	Solution Set (SS)	292

Annex A (normative): Cell state handling	278

A.1	Relation between the administrative state and the "Pre-operation state of the gNB-DU Cell"	278

A.2	Combined state diagram for gNB cell	278

Annex B (normative): NetworkSlice and NetworkSliceSubnet state handling	281

B.1	NetworkSlice instance state handling	281

B.2	State handling of NetworkSliceSubnet instance	282

Annex C (normative): Void	284

Annex D (normative): OpenAPI definition of the NR NRM	285

D.1	General	285

D.2	Void	285

D.3	Void	285

D.4	Solution Set (SS) definitions	285

D.4.1	Void	285

D.4.2	Void	285

D.4.3	OpenAPI document "TS28541_NrNrm.yaml"	285

Annex E (normative): YANG definitions for NR NRM	313

E.1	General	313

E.2	Void	313

E.3	Void	313

E.4	Void	313

E.5	Modules	313

E.5.1	module _3gpp-nr-nrm-beam.yang	313

E.5.1a	module _3gpp-nr-nrm-bwp.yang	315

E.5.1b	module _3gpp-nr-nrm-commonbeamformingfunction.yang	316

E.5.2	module _3gpp-nr-nrm-ep.yang	317

E.5.3	module _3gpp-nr-nrm-eutrancellrelation.yang	320

E.5.4	module _3gpp-nr-nrm-eutranetwork.yang	323

E.5.5	module _3gpp-nr-nrm-eutranfreqrelation.yang	323

E.5.6	module _3gpp-nr-nrm-eutranfrequency.yang	326

E.5.7	module _3gpp-nr-nrm-externalamffunction.yang	327

E.5.8	module _3gpp-nr-nrm-externalenbfunction.yang	328

E.5.9	module _3gpp-nr-nrm-externaleutrancell.yang	329

E.5.10	module _3gpp-nr-nrm-externalgnbcucpfunction.yang	331

E.5.11	module _3gpp-nr-nrm-externalgnbcuupfunction.yang	332

E.5.12	module _3gpp-nr-nrm-externalgnbdufunction.yang	333

E.5.13	module _3gpp-nr-nrm-externalnrcellcu.yang	334

E.5.14	module _3gpp-nr-nrm-externalservinggwfunction.yang	335

E.5.15	module _3gpp-nr-nrm-externalupffunction.yang	336

E.5.16	module _3gpp-nr-nrm-gnbcucpfunction.yang	337

E.5.17	module _3gpp-nr-nrm-gnbcuupfunction.yang	339

E.5.18	module _3gpp-nr-nrm-gnbdufunction.yang	341

E.5.19	module _3gpp-nr-nrm-nrcellcu.yang	343

E.5.20	module _3gpp-nr-nrm-nrcelldu.yang	345

E.5.21	module _3gpp-nr-nrm-nrcellrelation.yang	350

E.5.22	module _3gpp-nr-nrm-nrfreqrelation.yang	352

E.5.23	module _3gpp-nr-nrm-nrfrequency.yang	356

E.5.24	module _3gpp-nr-nrm-nrnetwork.yang	357

E.5.25	module _3gpp-nr-nrm-nrsectorcarrier.yang	357

E.5.26	module _3gpp-nr-nrm-rrmpolicy.yang	359

E.5.27	Void	361

E.5.28	module _3gpp-nr-nrm-danrmanagementfunction.yang	361

E.5.29	module _3gpp-nr-nrm-desmanagementfunction.yang	362

E.5.30	module _3gpp-nr-nrm-drachoptimizationfunction.yang	365

E.5.31	module _3gpp-nr-nrm-dmrofunction.yang	366

E.5.32	module _3gpp-nr-nrm-dpciconfigurationfunction.yang	368

E.5.33	module _3gpp-nr-nrm-cpciconfigurationfunction.yang	369

E.5.34	module _3gpp-nr-nrm-cesmanagementfunction.yang	370

E.5.35	module _3gpp-nr-nrm-operatordu.yang	373

E.5.36	module _3gpp-nr-nrm-nroperatorcelldu.yang	374

E.5.37	module _3gpp-nr-nrm-dlbofunction.yang	375

E.5.38	module _3gpp-nr-nrm-rimrsset.yang	376

E.5.39_module  3gpp-nr-nrm-bwpset.yang	383

E.6	Void	383

E.7	Mount information	384

Annex F (normative): Void	385

Annex G (normative): OpenAPI definition of the 5GC NRM	386

G.1	General	386

G.2	Void	386

G.3	Void	386

G.4	Solution Set (SS) definitions	386

G.4.1	Void	386

G.4.2	Void	386

G.4.3	OpenAPI document "TS28541_5GcNrm.yaml"	386

Annex H (normative): YANG definitions for 5GC	423

H.1	General	423

H.2	Void	423

H.3	Void	423

H.4	Void	423

H.5	Modules	423

H.5.1	module _3gpp-5g-common-yang-types.yang	423

H.5.1a	module _3gpp-5gc-nrm-affunction.yang	425

H.5.2	module _3gpp-5gc-nrm-amffunction.yang	426

H.5.3	module _3gpp-5gc-nrm-amfregion.yang	427

H.5.4	module _3gpp-5gc-nrm-amfset.yang	428

H.5.5	module _3gpp-5gc-nrm-ausffunction.yang	429

H.5.6	module _3gpp-5gc-nrm-dnfunction.yang	430

H.5.7	module _3gpp-5gc-nrm-ep.yang	431

H.5.8	module _3gpp-5gc-nrm-externalnrffunction.yang	439

H.5.9	module _3gpp-5gc-nrm-externalnssffunction.yang	440

H.5.10	module _3gpp-5gc-nrm-lmffunction.yang	441

H.5.11	module _3gpp-5gc-nrm-n3iwffunction.yang	442

H.5.12	module _3gpp-5gc-nrm-nfprofile.yang	442

H.5.13	module _3gpp-5gc-nrm-nfservice.yang	457

H.5.14	module _3gpp-5gc-nrm-ngeirfunction.yang	461

H.5.15	module _3gpp-5gc-nrm-nrffunction.yang	462

H.5.16	module _3gpp-5gc-nrm-nssffunction.yang	464

H.5.17	module _3gpp-5gc-nrm-nwdaffunction.yang	465

H.5.18	module _3gpp-5gc-nrm-pcffunction.yang	466

H.5.19	module _3gpp-5gc-nrm-seppfunction.yang	467

H.5.19a	 module _3gpp-5gc-nrm- externalseppfunction.yang	468

H.5.20	module _3gpp-5gc-nrm-smffunction.yang	469

H.5.21	module _3gpp-5gc-nrm-smsffunction.yang	471

H.5.22	module _3gpp-5gc-nrm-udmfunction.yang	472

H.5.23	module _3gpp-5gc-nrm-udrfunction.yang	473

H.5.24	module _3gpp-5gc-nrm-udsffunction.yang	474

H.5.25	module _3gpp-5gc-nrm-upffunction.yang	475

H.5.26	module _3gpp-5gc-nrm-scpfunction.yang	476

H.5.27	module _3gpp-5gc-nrm-neffunction.yang	477

H.5.28	module _3gpp-5gc-nrm-QFQoSMonitoringControl.yang	478

H.5.29	module _3gpp-5gc-nrm-GtpUPathQoSMonitoringControl.yang	480

H.5.30	module _3gpp-5gc-nrm-configurable5QISet.yang	481

H.5.31	module _3gpp-5gc-nrm-FiveQiDscpMappingSet.yang	483

H.5.32	module _3gpp-5gc-nrm-PredefinedPccRuleSet.yang	484

H.5.33	module _3gpp-5gc-nrm-dynamic5QISet.yang	493

H.6	Void	494

H.7	Mount information	494

Annex I (normative): Void	495

Annex J (normative): OpenAPI definition of the Slice NRM	496

J.1	General	496

J.2	Void	496

J.3	Void	496

J.4	Solution Set (SS) definitions	496

J.4.1	Void	496

J.4.2	Void	496

J.4.3	OpenAPI document "TS28541_SliceNrm.yaml"	496

Annex K (normative): Void	508

Annex L (normative): Relation of GSMA GST, ServiceProfile and SliceProfile	509

L.1	General	509

L.2	GSMA GST, ServiceProfile and sliceProfile	509

Annex M (normative): Managed NF Service state handling	511

M.1	Combined state diagram for a Managed NF Service	511

Annex N (normative): YANG definition of the Slice NRM	513

N.1	General	513

N.2	Modules	513

N.2.1	module _3gpp-ns-nrm-networkslice.yang	513

N.2.2	module _3gpp-ns-nrm-networkslicesubnet.yang	514

N.2.3	Void	517

N.2.4	module _3gpp-ns-nrm-serviceprofile.yang	517

N.2.5	module _3gpp-ns-nrm-sliceprofile.yang	524

N.2.6	module _3gpp-ns-nrm-common.yang	537

Annex O (informative): Change history	539

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

The present document is part of a TS-family covering the 3rd Generation Partnership Project Technical Specification Group Services and System Aspects Management and orchestration of networks, as identified below:

TS 28.540:	Management and orchestration of 5G networks; Network Resource Model (NRM); Stage 1.

TS 28.541:	Management and orchestration of 5G networks; Network Resource Model (NRM); Stage 2 and stage 3.

# 1	Scope

The present document specifies the Information Model and Solution Set for the Network Resource Model (NRM) definitions of NR, NG-RAN, 5G Core Network (5GC) and network slice, to fulfil the requirements identified in TS 28.540 [10].

The Information Model defines the semantics and behaviour of information object class attributes and relations visible on the management interfaces in a protocol and technology neutral way. And Solution Set defines one or more solution set(s) with specific protocol(s) according to the Information Model definitions.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[4]	3GPP TS 38.401: "NG-RAN; Architecture description".

[5]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[6]	3GPP TS 38.420: "NG-RAN; Xn general aspects and principles".

[7]	3GPP TS 38.470: "NG-RAN; F1 general aspects and principles".

[8]	3GPP TS 38.473: "NG-RAN; F1 application protocol (F1AP)".

[9]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage 2".

[10]	3GPP TS 28.540: "Management and orchestration; 5G Network Resource Model (NRM);Stage 1".

[11]	3GPP TS 28.662: "Telecommunication management; Generic Radio Access Network (RAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS) ".

[12]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[13]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[14]	3GPP TS 36.410: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 general aspects and principles".

[15]	3GPP TS 36.423: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 application protocol".

[16]	3GPP TS 36.425: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 interface user plane protocol".

[17]	3GPP TS 28.625: "State Management Data Definition Integration Reference Point (IRP); Information Service (IS)".

[18]	ITU-T Recommendation X.731: "Information technology - Open Systems Interconnection - Systems Management: State management function".

[19]	3GPP TS 28.658: "Telecommunications management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)".

[20]	3GPP TS 28.702: "Core Network (CN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[21]	3GPP TS 28.708: "Telecommunication management; Evolved Packet Core (EPC) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)".

[22]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[23]	3GPP TS 29.510: "5G system; Network Function Repository Services; Stage 3".

[24]	3GPP TS 29.531: "5G System; Network Slice Selection Services Stage 3".

[25]	Void.

[26]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[27]	3GPP TS 28.554: "Management and orchestration; 5G End to end Key Performance Indicators (KPI)".

[28]	3GPP TS 22.261: "Service requirements for next generation new services and markets".

[29]	ETSI GS NFV-IFA 013 V2.4.1 (2018-02) "Network Function Virtualisation (NFV); Management and Orchestration; Os-Ma-nfvo Reference Point - Interface and Information Model Specification".

[30]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[31]	Void.

[32]	3GPP TS 38.211: "NR; Physical channels and modulation".

[33]	3GPP TS 32.616: "Telecommunication management; Configuration Management (CM); Bulk CM Integration Reference Point (IRP); Solution Set (SS) definitions".

[34]	Void

[35]	3GPP TS 28.532: "Management and orchestration; Management services".

[36]	Void.

[37]	IETF RFC 791: "Internet Protocol".

[38]	IETF RFC 2373: "IP Version 6 Addressing Architecture".

[39]	IEEE 802.1Q: "Media Access Control Bridges and Virtual Bridged Local Area Networks".

[40]	ETSI GR NFV-IFA 015 (V2.4.1): "Network Function Virtualisation (NFV) Release 2; Management and Orchestration; Report on NFV Information Model".

[41]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[42]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[43]	3GPP TS 32.156: "Telecommunication management; Fixed Mobile Convergence (FMC) model repertoire".

[44]	IETF RFC 4122: "A Universally Unique IDentifier (UUID) URN Namespace".

[45]	IETF RFC 8528: "YANG Schema Mount".

[46]	Void

[47]	3GPP TS 32.160: "Management and orchestration; Management Service Template".

[48]	3GPP TS 38.463: "NG-RAN; E1 application protocol (E1AP)".

[49]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in Idle mode and RRC Inactive state".

[50]	GSMA NG.116 - Generic Network Slice Template Version 3.0 (2020-05-22).

[51]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains; Stage 1".

[52]	3GPP TS 33.501: "Security architecture and procedures for the 5G System".

[53]	3GPP TS 38.901: "Study on channel model for frequencies from 0.5 to 100 GHz ".

[54]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[55]	3GPP TS 38.215: "NR; Physical layer measurements".

[56]	3GPP TS 29.244: "Technical Specification Group Core Network and Terminals; Interface between the Control Plane and the User Plane Nodes; Stage 3".

[57]	3GPP TS 28.313: "Self-Organizing Networks (SON) for 5G networks".

[58]	3GPP TS 38.423: "NR; Xn application protocol (XnAP)".

[59]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[60]	3GPP TS 29.512: "5G System; Session Management Policy Control Service; Stage 3".

[61]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[62]	3GPP TS 29.214: "Policy and Charging Control over Rx reference point".

[63]	IETF RFC 7042: "IANA Considerations and IETF Protocol and Documentation Usage for IEEE 802 Parameters".

[64]	IEEE 802.3-2015: "IEEE Standard for Ethernet".

[65]	IEEE 802.1Q-2014: "Bridges and Bridged Networks".

[66]	IETF RFC 4301: "Security Architecture for the Internet Protocol".

[67]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[68]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[69]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[70]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[71]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[72]	3GPP TS 28.705: "Telecommunication management; IP Multimedia Subsystem (IMS) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[73]	3GPP TS 23.304: " Proximity based Services (ProSe) in the 5G System".

[74]	IETF RFC 8436: " Update to IANA Registration Procedures for Pool 3 Values in the Differentiated Services Field Codepoints (DSCP) Registry".

[75]	ECMA-262: "ECMAScript® Language Specification", https://www.ecma-international.org/ecma-262/5.1/.

[76]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[77]	IANA: "SMI Network Management Private Enterprise Codes", http://www.iana.org/assignments/enterprise-numbers.

[78]	3GPP TS 23.548:" 5G System Enhancements for Edge Computing; Stage 2".

[79]	3GPP TS 28.538: "Edge Computing Management".

[80]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[81]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[82]	IETF RFC 5952: "A recommendation for IPv6 address text representation".

[83]	IETF RFC 8299: "YANG Data Model for L3VPN Service Delivery".

[84]	IETF RFC 8466: "A YANG Data Model for Layer 2 Virtual Private Network (L2VPN) Service Delivery".

[85]	3GPP TS 29.520: "5G System; Network Data Analytics Services; Stage 3".

[86]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3".

[87]	3GPP TS 29.517: "5G System; Application Function Event Exposure Service; Stage 3".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1], TS 28.540 [10] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1] and TS 28.540 [10].

## 3.2	Symbols

void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], TS 23.501 [2], TS 38.401 [4], TS 28.540 [10] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] , TS 23.501 [2], TS 38.401 [4] and TS 28.540 [10].

BWP	Bandwidth part

CHO	Conditional Handover

CM	Configuration Management

DAPS	Dual Active Protocol Stack

DN	Distinguished Name

IOC	Information Object Class

JSON	JavaScript Object Notation

NFV	Network Functions Virtualisation

NRM	Network Resource Model

NS	Network Service

NSI	Network Slice Instance

NSSAI	Network Slice Selection Assistance Information

NSSI	Network Slice Subnet Instance

PNF	Physical Network Function

RIM	Remote interference management

RIM-RS	Remote interference management reference signal

SBA	Service Based Architecture

SS	Solution Set

TN	Transport Network

VNF	Virtualised Network Function