| 3GPP TR 28.878 V0.1.0 (2024-04) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Management of Network Sharing Phase3; (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	7

4	Concepts and background	7

5	Use Cases	7

5.1	Use Case#1: Service-based management architecture and access requirements for MOCN	7

5.1.1	Descriptions	7

5.1.2	Potential requirements	7

5.2	Use Case#2: Trace job and collection requirements for POPs	7

5.2.1	Descriptions	7

5.2.2	Potential requirements	8

6	Conclusions and recommendations	8

Annex <X> (informative): Change history	9

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document study the management of network sharing including Trace job and collection requirements for POPs, some issues related Rel-18, such as enhancements for SBMA based MOCN scenario, scenario related to access rights on CRUD operation of MOCN management data, and potential enhancements for Indirect Network Sharing scenarios..

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system".

[3]	3GPP TR 22.851: "Feasibility Study on Network Sharing Aspect".

[4]	3GPP TS 32.130: "Network sharing; Concepts and requirements".

[5] 	3GPP TR 28.835: "Study on Management Aspect Enhancement of 5G MOCN Network Sharing Phase2".

[6] 	3GPP TS 28.622: "Generic Network Resource Model (NRM);Integration Reference Point (IRP); Information Service (IS)".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Concepts and background

Network sharing is a crucial approach to decrease CAPEX of 5G constructions for operators. As stated in TS 22.261[2] the increased density of access nodes needed to meet future performance objectives pose considerable challenges in deployment and acquiring spectrum and antenna locations. RAN sharing is seen as a technical solution to these issues. Plenty of previous normative works have been completed during Rel-17 and Rel-18.

During Rel-18, SA5 has specified new scenarios for MOCN Network sharing, SBMA deployment examples for MOCN management architectures and related NRM enhancements in TS 32.130[4]. Some leftover issues for existing MOCN, including Trace job and collection requirements for POPs, etc., are supposed to be further studied.

# 5 	Use Cases

## 5.1	Use Case#1: Service-based management architecture and access requirements for MOCN

### 5.1.1	Description

In TR 28.835[X] of Rel 18, there are some remaining issues that need to continue to study, including Issue #4: Service-based management architecture for MOCN and Issue #5: Use of MSAC for MOCN sharing scenarios.

The Service Based Management Architecture (SBMA) need to be provided for MOCN networking sharing. But the potential solution is an example of the service-based management architecture for MOCN, and the comprehensive service-based management architecture needs further study.

The management system of MOP should have the capability to allow authorized POP users to access, read and download data of configuration, performance measurements and alarms. Therefore, some potential solutions of users’ access and relevant system user permissions for each POP need to be studied.

### 5.1.2	Potential requirements

REQ-NetShare_Config-1: The MOP MnS producer should support MnS consumer to request to access the MOP management system.

REQ-NetShare_Config-2: The MOP MnS producer should allow or forbid MnS consumer to obtain configuration, performance measurements and alarm data.

REQ-NetShare_Config-3: The MOP MnS producer should support roles for each POP as MnS consumers.

## 5.2	Use Case#2: Trace job and collection requirements for POPs

### 5.2.1	Description

For 5G MOCN Networking Sharing, different POPs require different data for personalized operation and maintenance services, including collecting different trace and performance data requirements. For example, the different trace data or performanced data with collection time window, collection requirements, etc., should be reported for each POP.

The existing specifications do not provide some descriptions of trace job, collection flows and interface details for NG-RAN MOCN network sharing scenario. Therefore, some potential solutions of trace job and collection requirements for POPs need to be studied.

### 5.2.2	Potential requirements

REQ-NetShare_TraceConfig-1: The 3GPP management system of the MOP should have the capability to support operator-specific management data collection jobs.

6	Conclusions and recommendations

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-04 | SA5#154 |  |  |  |  | Initial skeleton | 0.0.0 |
| 2024-04 | SA5#154 | S5-242132 |  |  |  | pCR TR 28.878 Add scope | 0.1.0 |
| 2024-04 | SA5#154 | S5-242133 |  |  |  | pCR TR 28.878 Add structure | 0.1.0 |
| 2024-04 | SA5#154 | S5-242134 |  |  |  | pCR TR 28.878 Concepts and overview | 0.1.0 |
| 2024-04 | SA5#154 | S5-242135 |  |  |  | pCR TR 28.878 Add use case, requirements for Service-based management architecture and access requirements for MOCN | 0.1.0 |
| 2024-04 | SA5#154 | S5-242136 |  |  |  | pCR TR 28.878 Add use case, requirements for Trace job and collection requirements for POPs | 0.1.0 |