| 3GPP TS 29.176 V0.2.0 (2023-05) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; IP Multimedia Subsystems (IMS); Data Channel Media Function (DCMF) Services; Stage 3 (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions, symbols and abbreviations	8

3.1	Definitions	8

3.2	Symbols	8

3.3	Abbreviations	8

4	Overview	8

5	Services offered by the DCMF	9

5.1	Introduction	9

5.2	Ndcmf_MediaResourceManagement (MRM) Service	9

5.2.1	Service Description	9

5.2.2	Service Operations	9

5.2.2.1	Introduction	9

5.2.2.2	Ndcmf_MRM_Create Service Operation	9

5.2.2.2.1	General	9

5.2.2.2.2	Creation of a new media context	10

5.2.2.3	Ndcmf_MRM_Update Service Operation	11

5.2.2.3.1	General	11

5.2.2.3.2	Updating an existing media context	11

5.2.2.4	Ndcmf_MRM_Delete Service Operation	12

5.2.2.4.1	General	12

5.2.2.4.2	Deleting an existing media context	12

5.3	<Service 2 > Service	12

6	API Definitions	12

6.1	Ndcmf_MRM Service API	12

6.1.1	Introduction	12

6.1.2	Usage of HTTP	13

6.1.2.1	General	13

6.1.2.2	HTTP standard headers	13

6.1.2.2.1	General	13

6.1.2.2.2	Content type	13

6.1.2.3	HTTP custom headers	13

6.1.3	Resources	13

6.1.3.1	Overview	13

6.1.3.2	Resource: Contexts Collection	14

6.1.3.2.1	Description	14

6.1.3.2.2	Resource Definition	14

6.1.3.2.3	Resource Standard Methods	14

6.1.3.2.4	Resource Custom Operations	15

6.1.3.3	Resource: Individual Context	15

6.1.3.3.1	Description	15

6.1.3.3.2	Resource Definition	15

6.1.3.3.3	Resource Standard Methods	15

6.1.3.3.4	Resource Custom Operations	17

6.1.4	Custom Operations without associated resources	17

6.1.5	Notifications	17

6.1.6	Data Model	17

6.1.6.1	General	17

6.1.6.2	Structured data types	18

6.1.6.2.1	Introduction	18

6.1.6.2.2	Type: MediaContext	18

6.1.6.2.3	Type: TerminationInfo	18

6.1.6.2.4	Type: MediaInfo	19

6.1.6.2.5	Type: Endpoint	22

6.1.6.2.6	Type: DcStream	23

6.1.6.3	Simple data types and enumerations	23

6.1.6.3.1	Introduction	23

6.1.6.3.2	Simple data types	23

6.1.6.3.3	Enumeration: MediaResourceType	23

6.1.6.3.4	Enumeration: SecuritySetup	24

6.1.6.3.5	Enumeration: MediaProxy	24

6.1.7	Error Handling	24

6.1.7.1	General	24

6.1.7.2	Protocol Errors	24

6.1.7.3	Application Errors	24

6.1.8	Feature negotiation	25

6.1.9	Security	25

6.1.10	HTTP redirection	25

6.2	< Service 2> Service API	25

Annex A (normative): OpenAPI specification	26

A.1	General	26

A.2	Ndcmf_MRM API	26

A.3	<Service 2> API	29

Annex B (informative): Change history	29

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the stage 3 protocol and data model for the Ndcmf Service Based Interface. It provides stage 3 protocol definitions and message flows, and specifies the API for each service offered by the Data Channel Media Function (DCMF).

The 5G System stage 2 architecture and procedures are specified in TS 23.501 [2] and TS 23.502 [3].

The IP Multimedia Subsystem (IMS) Data Channel (DC) architecture and procedures are specified in annex AC of TS 23.228 [14].

The Technical Realization of the Service Based Architecture and the Principles and Guidelines for Services Definition are specified in TS 29.500 [4] and TS 29.501 [5].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[7]	3GPP TR 21.900: "Technical Specification Group working methods".

[8]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[9]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[10]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[11]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[12]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[13]	IETF RFC 7807: "Problem Details for HTTP APIs".

[14]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[15]	IETF RFC 6902: " JavaScript Object Notation (JSON) Patch".

[16]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces Stage 3".

[17]	IETF RFC 6901: "JavaScript Object Notation (JSON) Pointer".

[18]	IETF RFC 8122: "Connection-Oriented Media Transport over the Transport Layer Security (TLS) Protocol in the Session Description Protocol (SDP)".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

DC2	 Reference point between DCMF and IMS AS

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

DC	Data Channel

DCMF	Data Channel Media Function

IMS AS	IP Multimedia Subsystem Application Server

# 4	Overview

Within the IMS DC architecture, the DCMF offers services to the IMS AS via the Ndcmf service based interface (see 3GPP TS 23.228 [14]).

Figure 4.1-1 provides the reference model (in service-based interface representation and in reference point representation), with focus on the DCMF services.

Figure 4.1-1: Reference model – DCMF

DC2 is the reference point between DCMF and IMS AS.

# 5	Services offered by the DCMF

## 5.1	Introduction

The DCMF offers the following services via the Ndcmf interface:

-	Ndcmf_MediaResourceManagement (MRM) Service

Table 5.1-1 summarizes the corresponding APIs defined for this specification.

Table 5.1-1: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| Ndcmf_MRM | 6.1 | Ndcmf Media Resource Management Service | TS29176_Ndcmf_MRM.yaml | ndcmf-mrm | A.2 |

## 5.2	Ndcmf_MediaResourceManagement (MRM) Service

### 5.2.1	Service Description

The Ndcmf_MRM service as defined in 3GPP TS 23.228 [14] is provided by the Data Channel Media Function (DCMF). This service enables the consumer to create, update and delete media resources.

The media resource represents a media context including one or multiple media terminations. A media termination includes media resources for one or multiple medias on the Mb interface. When a media pass through the DCMF, there is one termination for the input stream and one termination for the output stream.

### 5.2.2	Service Operations

#### 5.2.2.1	Introduction

The Ndcmf_MRM service supports the following service operation.

Table 5.2.1-1: Service operations supported by the Ndcmf_MRM service

| Service Operations | Description | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Create | Create a new media context including one or multiple media terminations. | Request/Response | IMS AS |
| Update | Update the one or multiple existing media resources within a specific media context. | Request/Response | IMS AS |
| Delete | Delete a specific media context including all the existing terminations and medias. | Request/Response | IMS AS |

#### 5.2.2.2	Ndcmf_MRM_Create Service Operation

##### 5.2.2.2.1	General

The Ndcmf_MRM_Create service operation is used by an NF service consumer to create a media context including one or multiple terminations and reserve media resources for anchoring one or multiple medias of Mb interface in each termination on DCMF. The consumer NF may also include application function (e.g. DCSF, DC AS) specification information requested by the application layer to be applied on the media by the DCMF.

##### 5.2.2.2.2	Creation of a new media context

The NF service consumer shall request a new context by using HTTP method POST with "{apiRoot}/ndcmf/<apiVersion>/contexts" as resource URI representing the "Contexts Collection", see clause 6.1.3.2.

Figure 5.2.2.2.2-1 Creation of a media context

1.	The NF Service Consumer shall send a POST request to create an "Individual Context" resource in the DCMF. The payload body of the POST request shall contain a representation of the individual context resource to be created.

The NF service consumer shall include the following information in the HTTP message body:

-	List of termination descriptions;

-	List of media steam descriptions;

-	Media ID, i.e. a unique identity of the media stream within the media context instance;

-	Remote Mb specifications, i.e. the media stream IP address and port allocated at the remote endpoint, i.e. remote UE, remote network.

-	Media resource description, which includes

-	Media resource type, e.g. DC, AR.

-	If media resource type is set to "DC", the media resource description shall include:

-	media proxy configuration applicable to the media flow;

-	remote MDC1/MDC2 media specification information to be applied on the media by the DCMF;

-	Replacement HTTP URL allocated by the application layer representing the application list (e.g. graphical user interface) offered to the IMS subscriber via the MDC1 interface.

-	Data Channel Mapping and Configuration information when originating/terminating data channel media flows on the Mb interface.

-	SCTP stream Id for the DC;

-	subprotocol, order, maxRetry, maxTime and priority may be included.

-	Maximum Message Size, which represents the maximum size to be expected.

-	Data Channel Port, which represents the port of SCTP port for the Data Channel.

-	Security Setup, which identifies the security setup of the DTLS connection.

-	Security Certificate Fingerprint, which identifies the security certificate fingerprint.

-	Security Transport Identity, which identifies transport layer identity.

2a.	Upon the reception of the HTTP POST request, if the request is accepted and no error occur, the DCMF shall:

-	create a new media context;

-	assign a media contextId;

-	assign a terminationId for each termination descriptor; and

-	reserve media resources for each mediaId.

The DCMF shall include a HTTP Location header to provide the location of a newly created resource (MediaContext) together with the status code 201 indicating the requested resource is created in the response message.

2b.	On failure, one of the HTTP status code listed in Table 6.1.3.2.3.1-3 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.3.2.3.1-3.

#### 5.2.2.3	Ndcmf_MRM_Update Service Operation

##### 5.2.2.3.1	General

The Ndcmf_MRM_Update service operation is used by an NF service consumer to update one or multiple existing media resources within a specific media context on DCMF. Terminations and/or Medias can be added, modified, or deleted within an Update request.

##### 5.2.2.3.2	Updating an existing media context

The NF Service Consumer shall modify the context by using HTTP method PATCH with the URI "{apiRoot}/ndcmf/<apiVersion>/contexts/{contextId}" as resource URI representing the "Individual Context ", see clause 6.1.3.3.

Figure 5.2.2.3.2-1 Update a media context

1.	The NF Service Consumer shall send a PATCH request to modify a context resource in the DCMF. The modification may be for adding, updating or deleting terminations of the existing media context.

For adding, removing or updating a termination in an existing MediaContext, the payload body of the PATCH request shall contain an "add", "remove" or "replace" PATCH operation respectively, with one item of the attribute "terminations" of the MediaContext.

2a.	On success, if the change is to delete the existing termination and DCMF accepts the request change, the DCMF shall return the status code 204 No Content.

2b. On success, if the change is to add a new termination or to update the existing termination and DCMF accepts the request change, the DCMF shall return the status code 200 OK. The response shall contain the new resource representation of the resource Individual Context, which includes the added and modified resource or its sub-resource.

2c	On failure, one of the HTTP status code listed in Table 6.1.3.3.3.1-3 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in Table 6.1.3.3.3.1-3.

#### 5.2.2.4	Ndcmf_MRM_Delete Service Operation

##### 5.2.2.4.1	General

The Ndcmf_MRM_Delete service operation is used by an NF service consumer to delete an existing media context including all existing terminations and medias on DCMF previously created by itself.

##### 5.2.2.4.2	Deleting an existing media context

The NF Service Consumer shall delete an existing context by using HTTP method DELETE with the URI "{apiRoot}/ndcmf/<apiVersion>/contexts/{contextId}" as resource URI representing the "Individual Context ", see clause 6.1.3.3.

Figure 5.2.2.4.2-1 Delete a media context

1.	The NF Service Consumer shall send a DELETE request to delete an existing context resource in the DCMF.

2a.	On success, the request is accepted, the DCMF shall reply with the status code 204 No Content indicating the resource identified by context ID is successfully deleted in the response message.

2b.	On failure, one of the HTTP status code listed in Table 6.1.3.3.3.2-3 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in Table 6.1.3.3.3.2-3.

## 5.3	<Service 2 > Service

And so on if there are more than two services offered by the NF. Same structure as in clause 5.2.

# 6	API Definitions

## 6.1	Ndcmf_MRM Service API

### 6.1.1	Introduction

The Ndcmf_MRM service shall use the Ndcmf_MRM API.

The API URI of the Ndcmf_MRM API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests from the NF service consumer towards the NF service producer shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [5].

-	The <apiName> shall be " ndcmf_mrm".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 6.1.3.

### 6.1.2	Usage of HTTP

#### 6.1.2.1	General

HTTP/2, IETF RFC 7540 [11], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

The OpenAPI [6] specification of HTTP messages and content bodies for the Ndcmf_MRM API is contained in Annex A.

#### 6.1.2.2	HTTP standard headers

##### 6.1.2.2.1	General

See clause 5.2.2 of 3GPP TS 29.500 [4] for the usage of HTTP standard headers.

##### 6.1.2.2.2	Content type

JSON, IETF RFC 8259 [12], shall be used as content type of the HTTP bodies specified in the present specification as specified in clause 5.4 of 3GPP TS 29.500 [4]. The use of the JSON format shall be signalled by the content type "application/json".

"Problem Details" JSON object shall be used to indicate additional details of the error in a HTTP response body and shall be signalled by the content type "application/problem+json", as defined in IETF RFC 7807 [13].

The use of JSON Patch (IETF RFC 6902 [15]) format in a HTTP request body shall be signaled by the content type "application/json-patch+json".

#### 6.1.2.3	HTTP custom headers

The mandatory HTTP custom header fields specified in clause 5.2.3.2 of 3GPP TS 29.500 [4] shall be supported, and the optional HTTP custom header fields specified in clause 5.2.3.3 of 3GPP TS 29.500 [4] may be supported.

### 6.1.3	Resources

#### 6.1.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.1.3.1-1 depicts the resource URIs structure for the Ndcmf_MRM API.

Figure 6.1.3.1-1: Resource URI structure of the Ndcmf_MRM API

Table 6.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.3.1-1: Resources and methods overview

| Resource purpose/name | Resource URI (relative path after API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Contexts Collection | /contexts | POST | Creates a new Individual Context resource. |
| Individual Context | /contexts/{contextId} | PATCH | Updates an existing Individual Context resource identified by {contextId} |
|  |  | DELETE | Deletes an existing Individual Context resource identified by {contextId}. |

#### 6.1.3.2	Resource: Contexts Collection

##### 6.1.3.2.1	Description

The Contexts Collection resource represents a collection of contexts created by NF service consumers of Ndcmf_MRM service at a given DCMF. The resource is modelled as Collection resource archetype (see clause C.2 of 3GPP TS 29.501 [2]).

##### 6.1.3.2.2	Resource Definition

Resource URI: {apiRoot}/ ndcmf_mrm/<apiVersion>/contexts

This resource shall support the resource URI variables defined in table 6.1.3.2.2-1.

Table 6.1.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |

##### 6.1.3.2.3	Resource Standard Methods

6.1.3.2.3.1	POST

This method shall support the URI query parameters specified in table 6.1.3.2.3.1-1.

Table 6.1.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.3.2.3.1-3.

Table 6.1.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MediaContext | M | 1 | Creates a new Individual Context resource |

Table 6.1.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MediaContext | M | 1 | 201 Created | The creation of an Individual Media Context resource is confirmed and a representation of that resource is returned. |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 6.1.3.2.3.1-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/ndcmf-mrm/<apiVersion>/contexts/{contextId}. |

##### 6.1.3.2.4	Resource Custom Operations

None in the release of this specification.

#### 6.1.3.3	Resource: Individual Context

##### 6.1.3.3.1	Description

The Individual Context resource represents an individual context created by the NF service consumers of Ndcmf_MRM service at a given DCMF. This resource is modelled as the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [2]).

##### 6.1.3.3.2	Resource Definition

Resource URI: {apiRoot}/ndcmf_mrm/<apiVersion>/contexts/{contextId}

The <apiVersion> shall be set as described in clause 6.3.1.

This resource shall support the resource URI variables defined in table 6.1.3.3.2-1.

Table 6.1.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |
| contextId | string | Identifies an individual context to the Ndcmf_MRM service. |

##### 6.1.3.3.3	Resource Standard Methods

6.1.3.3.3.1	PATCH

This method shall support the URI query parameters specified in table 6.1.3.3.3.1-1.

Table 6.1.3.3.3.1-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.1-2 and the response data structures and response codes specified in table 6.1.3.3.3.1-3.

Table 6.1.3.3.3.1-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(PatchItem) | M | 1..N | Document describes the modification(s) to an Individual Context resource. |

Table 6.1.3.3.3.1-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MediaContext | M | 1..N | 200 OK | Represents a successful update on the media context. |
| n/a |  |  | 204 No Content |  |
| ProblemDetails | O | 0..1 | 403 Forbidden | Indicates the modification of media context has failed due to application error. The "cause" attribute may be used to indicate one of the following application errors: - MEDIA_ID_CONFLICT - MEDIA_CONNECTION_CHANGED |
| ProblemDetails | O | 0..1 | 404 Not Found | Indicates the modification of media context has failed due to application error. The "cause" attribute may be used to indicate one of the following application errors: - CONTEXT_NOT_FOUND |
| NOTE 1: The manadatory HTTP error status code for the PATCH method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

6.1.3.3.3.2	DELETE

This method shall support the URI query parameters specified in table 6.1.3.3.3.2-1.

Table 6.1.3.3.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.2-2 and the response data structures and response codes specified in table 6.1.3.3.3.2-3.

Table 6.1.3.3.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.3.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No content |  |
| Problem Details | O | 0..1 | 404 Not Found | Indicates the deletion of the context has failed due to application error. The "cause" attribute may be used to indicate one of the following application errors: - CONTEXT_NOT_FOUND |
| NOTE 1: The manadatory HTTP error status code for the DELETE method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

##### 6.1.3.3.4	Resource Custom Operations

None in the release of specification.

### 6.1.4	Custom Operations without associated resources

None in this release of specification.

### 6.1.5	Notifications

None in this release of specification.

### 6.1.6	Data Model

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Ndcmf_MRM  service based interface protocol.

Table 6.1.6.1-1: Ndcmf_MRM specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| MediaContext | 6.1.6.2.2 | Represents an individual context resource on DCMF. |  |
| TerminationInfo | 6.1.6.2.3 | Represents the termination information. |  |
| MediaInfo | 6.1.6.2.4 | Represents the media Information. |  |
| Endpoint | 6.1.6.2.5 | Represents the endpoint information of the Mb interface. |  |
| DcStream | 6.1.6.2.6 | Represents the SCTP stream information of the DC. |  |
| MediaType | 6.1.6.3.3 | Enumeration for the media type. |  |
| SecuritySetup | 6.1.6.3.4 | Enumeration for the security setup. |  |
| MediaProxy | 6.1.6.3.5 | Enumeration for the media configuration proxy. |  |

Table 6.1.6.1-2 specifies data types re-used by the Ndcmf_MRM service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Ndcmf_MRM service based interface.

Table 6.1.6.1-2: Ndcmf_MRM re-used Data Types

| Data type | Reference |  | Comments |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- |
| IpAddr | 3GPP TS 29.571 [16] |  |  |  |  |  |
| Uri |  | 3GPP TS 29.571 [16] |  |  |  |  |
| ProblemDetails |  | 3GPP TS 29.571 [16] |  | Problem Details |  |  |

#### 6.1.6.2	Structured data types

##### 6.1.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 6.1.6.2.2	Type: MediaContext

Table 6.1.6.2.2-1: Definition of type MediaContext

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| contextId | string | C | 0..1 | Identifies a specific context. It is assigned by the DCMF when creating a new context, i.e. Ndcmf_MRM_Create operation. |  |
| terminations | array(TerminationInfo) | M | 1..N | Represents the media terminations |  |

##### 6.1.6.2.3	Type: TerminationInfo

Table 6.1.6.2.3-1: Definition of type TerminationInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| terminationId | string | C | 0..1 | Identifies a specific media termination. It is assigned by the DCMF when creating a new termination. It shall be included in the response of adding a new media termination, i.e., the Ndcmf_MRM_Create operation and in the Ndcmf_MRM_Update operation with adding a new termination. |  |
| medias | array(MediaInfo) | M | 1..N | Represent the list of media resources for one or multiple medias on the Mb interface. |  |

##### 6.1.6.2.4	Type: MediaInfo

Table 6.1.6.2.4-1: Definition of type MediaInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mediaId | string | M | 1 | Identifies a media stream, e.g. data channel. |  |
| mediaResourceType | MediaResourceType | M | 1 | Represents the media resource type, e.g. DC, AR. |  |
| localMbEndpoint | Endpoint | C | 0..1 | Represents the local IP address and port number of the local endpoint of DCMF on the Mb interface. It will be allocated by DCMF. It shall be contained in the response when creating a new media stream, i.e. Ndcmf_MRM_Create operation or Ndcmf_MRM_Update operation with adding a new termination or adding a media in an existing termination. (NOTE 1) |  |
| remoteMbEndpoint | Endpoint | C | 0..1 | Represents the IP address and port number of the remote endpoint on the Mb interface, e.g. remote UE. It shall be contained in the request of creating a new media stream, i.e. Ndcmf_MRM_Create operation or Ndcmf_MRM_Update operation with adding a new termination or adding a media in an existing termination. It can be Null when originating a new media. (NOTE 1) |  |
| mediaProxyConfig | MediaProxy | C | 0..1 | Represents the media proxy configuration. It shall be contained if the mediaResourceType is set to "DC". For bootstrap data channel. |  |
| repalceHttpUrl | Uri | C | 0..1 | Represents URL for the specific IMS subscriber when requesting the application list (e.g. graphical user interface) via the MDC1 interface. It shall be included if the mediaResourceType is set to "DC" and the mediaId represents a bootstrap data channel and streamId is 0 or 100. |  |
| remoteMdc1Endpoint | Endpoint | C | 0..1 | Represents the remote MDC1 media endpoint information, i.e. the IP address and port number of DCSF. It shall be included if the mediaResourceType is set to "DC" and the mediaId represents a bootstrap data channel. |  |
| remoteMdc2Endpoint | Endpoint | C | 0..1 | Represents the remote MDC2 media endpoint address, i.e. the IP address and port number of DC AS. It shall be included if the mediaResourceType is set to "DC" and the mediaId represents a P2A/A2P application data channel and the mediaProxyConfig is set "HTTP". |  |
| mdc2Protocol | string | O | 0..1 | Represents the transport layer protocols for MDC2 interface. It may be included when mediaProxyConfig is set "HTTP" and remoteMdc2Endpoint is present. This IE is formatted as the following pattern, for example: 'UDP/DTLS/SCTP' |  |
| streams | array(DcStream) | C | 1..N | Represents the data channel mapping and configuration information when originating/terminating data channel media flows on the Mb interface. It shall be contained if the mediaResourceType is set "DC". |  |
| maxMessageSize | integer | O | 0..1 | Represents the maximum size of to be expected. It shall be contained if the mediaResourceType is set "DC". |  |
| localSctpPort | integer | C | 0..1 | Represents the port for the Data Channel. It will be allocated by DCMF. It shall be contained in the response when creating a new media stream, i.e. Ndcmf_MRM_Create operation or Ndcmf_MRM_Update operation with adding a new termination or adding a media in an existing termination and the mediaResourceType is set to "DC". (NOTE 1) |  |
| remoteSctpPort | integer | C | 0..1 | Represents the port for the Data Channel on the Mb interface. It shall be contained in the request when creating a new media stream , i.e. Ndcmf_MRM_Create operation or Ndcmf_MRM_Update operation with adding a new termination or adding a media in an existing termination and the mediaResourceType is set to "DC". (NOTE 1) |  |
| securitySetup | SecuritySetup | C | 0..1 | Represents the security set up of the DTLS association. It shall be contained if the mediaResourceType is set "DC". |  |
| localTlsId | string | C | 0..1 | Represents the local TLS ID on DCMF. It is allocated by DCMF for the media on Mb interface. It shall be contained in the response when creating a new media stream, i.e. Ndcmf_MRM_Create operation or Ndcmf_MRM_Update operation with adding a new termination or adding a media in an existing termination, and mediaResourceType is set "DC". (NOTE 1) |  |
| remoteTlsId | string | C | 0..1 | Represents the remote TLS ID for the media stream on the Mb interface. It shall be contained in the request when creating a new media stream i.e. Ndcmf_MRM_Create operation or Ndcmf_MRM_Update operation with adding a new termination or adding a media in an existing termination, and mediaResourceType is set "DC". (NOTE 1) |  |
| localFingerprint | string | C | 0..1 | Represents the certificate fingerprint for the local DTLS association. It is allocated by DCMF for the media on the Mb interface. It shall be contained in the response when creating a new media stream i.e. Ndcmf_MRM_Create operation or Ndcmf_MRM_Update operation with adding a new termination or adding a media in an existing termination, and mediaResourceType is set "DC". (NOTE 1) (NOTE 2) |  |
| remoteFingerprint | string | C | 0..1 | Represents the certificate fingerprint for the remote DTLS association on the Mb interface. It shall be contained in the request when creating a new media stream i.e. Ndcmf_MRM_Create operation or Ndcmf_MRM_Update operation with adding a new termination or adding a media in an existing termination, and mediaResourceType is set "DC". (NOTE 1) (NOTE 2) |  |
| NOTE 1: The IE cannot be changed once the media has been established. NOTE 2: The fingerprint is formatted as the pattern defined in IETF RFC 8122 [18]. For example: 'SHA-1 4A:AD:B9:B1:3F:82:18:3B:54:02:12:DF:3E:5D:49:6B:19:E5:7C:AB'. |  |  |  |  |  |

##### 6.1.6.2.5	Type: Endpoint

Table 6.1.6.2.5-1: Definition of type Endpoint

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ip | IpAddr | M | 0..1 | Represent the IP address of the endpoint |  |
| port | integer | M | 0..1 | Represent the UDP port of the endpoint. |  |

##### 6.1.6.2.6	Type: DcStream

Table 6.1.6.2.6-1: Definition of type MediaInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| streamId | integer | M | 1 | Identifies the SCTP stream. |  |
| subprotocol | string | O | 0..1 | Represents the subprotocol of the SCTP stream. It defaults to "http" if the mediaId represents bootstrap data channel. (NOTE 1) |  |
| order | boolean | O | 0..1 | Represents the stream is ordered or not, "true" for ordered delivery and "false" for unordered delivery. (NOTE 1) |  |
| maxRetry | integer | O | 0..1 | Represents the maximal number of the times a message will be retransmitted. Default value: 0 (NOTE 1) |  |
| maxTime | integer | O | 0..1 | Represents the maximal lifetime in milliseconds after which a message will no longer be transmitted or retransmitted. Default value: 0. (NOTE 1) |  |
| priority | integer | O | 0..1 | Represents the priority of data channel relative to other data channels. Default value: 256. (NOTE 1) |  |
| NOTE 1: The IE cannot be changed once the media has been established. |  |  |  |  |  |

#### 6.1.6.3	Simple data types and enumerations

##### 6.1.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.1.6.3.2	Simple data types

The simple data types defined in table 6.1.6.3.2-1 shall be supported.

Table 6.1.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 6.1.6.3.3	Enumeration: MediaResourceType

The enumeration MediaResourceType represents media resource type. It shall comply with the provisions defined in table 6.1.6.3.3-1.

Table 6.1.6.3.3-1: Enumeration MediaFunctionType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| DC | Represents the media resource type is DC. |  |
| AR | Represents the media resource type is AR. |  |

##### 6.1.6.3.4	Enumeration: SecuritySetup

The enumeration SecuritySetup represents the security setup of the DTLS connection. It shall comply with the previsions defined in table 6.1.6.3.4-1.

Table 6.1.6.3.4-1: Enumeration SecuritySetup

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| ACTIVE | Represents the endpoint will initiate an outgoing connection. |  |
| PASSIVE | Represents the endpoint will accept an incoming connection. |  |
| ACTPASS | Represents the endpoint is willing to accept an incoming connection or to initiate an outgoing connection. |  |

##### 6.1.6.3.5	Enumeration: MediaProxy

The enumeration MediaProxy represents the media proxy configuration applicable to the media flow. It shall comply with the previsions defined in table 6.1.6.3.5-1.

Table 6.1.6.3.5-1: Enumeration MediaProxy

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| HTTP | Represents the media proxy configuration is HTTP. |  |
| UDP | Represents the media proxy configuration is UDP. |  |

### 6.1.7	Error Handling

#### 6.1.7.1	General

For the Ndcmf_MRM API, HTTP error responses shall be supported as specified in clause 4.8 of 3GPP TS 29.501 [5]. Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [4] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [4].

In addition, the requirements in the following clauses are applicable for the Ndcmf_MRM API.

#### 6.1.7.2	Protocol Errors

No specific procedures for the Ndcmf_MRM service are specified.

#### 6.1.7.3	Application Errors

The application errors defined for the Ndcmf_MRM service are listed in Table 6.1.7.3-1.

Table 6.1.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| CONTEXT_NOT_FOUND | 404 Not Found | Indicates that the requested context is not found in the DCMF. |
| MEDIA_ID_CONFLICT | 403 Forbidden | Indicates that mediaId which identifies the newly added media has existed within the specific termination. |
| MEDIA_CONNECTION_CHANGED | 403 Forbidden | Indicates that the connection information which has been marked that cannot be changed once the media has established is changed in the request. including the remoteMbEndpoint, localMbEndpoint etc. |

### 6.1.8	Feature negotiation

The optional features in table 6.1.8-1 are defined for the Ndcmf_MRM API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 6.1.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 6.1.9	Security

As indicated in 3GPP TS 33.501 [8] and 3GPP TS 29.500 [4], the access to the Ndcmf_MRM API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [9]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [10]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming services offered by the Ndcmf_MRM API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [10], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Ndcmf_MRM service.

The Ndcmf_MRM API defines a single scope " ndcmf-mrm" for the entire service, and it does not define any additional scopes at resource or operation level.

### 6.1.10	HTTP redirection

An HTTP request may be redirected to a different <NF> service instance when using direct or indirect communications (see 3GPP TS 29.500 [4]).

An SCP that reselects a different <NF> producer instance will return the NF Instance ID of the new <NF> producer instance in the 3gpp-Sbi-Producer-Id header, as specified in clause 6.10.3.4 of 3GPP TS 29.500 [4].

If an <NF> redirects a service request to a different <NF> using an 307 Temporary Redirect or 308 Permanent Redirect status code, the identity of the new <NF> towards which the service request is redirected shall be indicated in the 3gpp-Sbi-Target-Nf-Id header of the 307 Temporary Redirect or 308 Permanent Redirect response as specified in clause 6.10.9.1 of 3GPP TS 29.500 [4].

## 6.2	< Service 2> Service API

And so on if there are more than two services supported by the NF. Same structure as in clause 6.1.

Annex A (normative):
OpenAPI specification

# A.1	General

This Annex specifies the formal definition of the API(s) defined in the present specification. It consists of OpenAPI specifications in YAML format.

This Annex takes precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API(s).

NOTE 1:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification files contained in this 3GPP Technical Specification are available on a Git-based repository that uses the GitLab software version control system (see clause 5.3.1 of 3GPP TS 29.501 [5] and clause 5B of 3GPP TR 21.900 [7]).

# A.2	Ndcmf_MRM API

One clause is introduced per Service, with the corresponding OpenAPI 3.0.0 Document.

Rapporteur of a new TS, which is based on this template shall inset a correct year below (see © <yyyy>). The rapporteur shall also update the year, if changes on the OpenAPI are agreed in a different year.

openapi: 3.0.0

info:

title: <API Name>

version: 1.0.0-alpha.1

description: |

<API Name> Service.

© <yyyy>, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.<xxx> V<x.y.z>; <TS Name>.

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.xxx/

servers:

- url: '{apiRoot}/<API name in lower letters. Composed names are separated with a hyphen>/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- <API name in lower letters with undesrscores>

paths:

# API specific definitions , below is an example

/subscriptions:

post:

summary: subscribe to notifications

operationId: CreateIndividualSubcription

tags:

- Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NsmfEventExposure'

responses:

'201':

description: Success

content:

application/json:

schema:

$ref: '#/components/schemas/<xxx>'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/<API name in lower letters. Composed names are separated with a hyphen>/<version>/subscriptions/{subId}'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

myNotification:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/<yyy>'

responses:

'204':

description: No Content, Notification was succesfull

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscriptions/{subId}:

get:

summary: retrieve subscription

operationId: GetIndividualSubcription

tags:

- IndividualSubscription (Document)

parameters:

- name: subId

in: path

description: Event Subscription ID

required: true

schema:

type: string

responses:

'200':

description: OK. Resource representation is returned

content:

application/json:

schema:

$ref: '#/components/schemas/<xxx>'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: update subscription

operationId: ReplaceIndividualSubcription

tags:

- IndividualSubscription (Document)

tags:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/<xxx>'

parameters:

- name: subId

in: path

description: Event Subscription ID

required: true

schema:

type: string

responses:

'200':

description: OK. Resource was succesfully modified and representation is returned

content:

application/json:

schema:

$ref: '#/components/schemas/<xxx>'

'204':

description: No Content. Resource was succesfully modified

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: unsubscribe from notifications

operationId: DeleteIndividualSubcription

tags:

- IndividualSubscription (Document)

parameters:

- name: subId

in: path

description: Event Subscription ID

required: true

schema:

type: string

responses:

'204':

description: No Content. Resource was succesfully deleted

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

<API name in lower letters . Composed names are separated with a hyphen>: Access to the <API Name> API

# API specific definitions

# A.3	<Service 2> API

And so on if there are more than two services supported by the NF.

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-04 | CT4#115e | C4-231552 |  |  |  | Version 0.1.0 | 0.1.0 |
| 2023-05 | CT4#116 | C4-232178 |  |  |  | Clause 4 Overview | 0.2.0 |
| 2023-05 | CT4#116 | C4-232224 |  |  |  | Clause 5.1 Introduction of DCMF services | 0.2.0 |
| 2023-05 | CT4#116 | C4-232506 |  |  |  | Service description and service operation introduction of MRM service | 0.2.0 |
| 2023-05 | CT4#116 | C4-232607 |  |  |  | MRM API definition | 0.2.0 |
| 2023-05 | CT4#116 | C4-232637 |  |  |  | MRM service operations | 0.2.0 |