| 3GPP TS 29.256 V0.5.0 (2021-11) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; 5G System; Uncrewed Aerial Systems Network Function (UAS-NF); Aerial Management Services; Stage 3 (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	8

4	Overview	8

4.1	Introduction	8

5	Services offered by the NEF(UAS-NF)	9

5.1	Introduction	9

5.2	Nnef_Authentication Service	9

5.2.2.3	AuthNotify Service Operation	11

5.2.2.3.1	General	11

6	API Definitions	12

6.1	Nnef_Authentication Service API	12

6.1.1	Introduction	12

6.1.2	Usage of HTTP	13

6.1.2.1	General	13

6.1.2.2	HTTP standard headers	13

6.1.2.2.1	General	13

6.1.2.2.2	Content type	13

6.1.2.3	HTTP custom headers	13

6.1.3	Resources	13

6.1.3.1	Overview	13

6.1.3.2	Resource: uav-authentications	14

6.1.3.2.1	Description	14

6.1.3.2.2	Resource Definition	14

6.1.3.2.3	Resource Standard Methods	14

6.1.3.2.3.1	POST	14

6.1.3.2.4	Resource Custom Operations	15

6.1.5	Notifications	16

6.1.5.1	General	16

6.1.5.2	Authentication Notification	16

6.1.5.2.1	Description	16

6.1.5.2.2	Target URI	16

6.1.5.2.3	Standard Methods	16

6.1.5.2.3.1	POST	16

6.1.6	Data Model	17

6.1.6.1	General	17

6.1.6.2	Structured data types	18

6.1.6.2.1	Introduction	18

6.1.6.2.2	Type: UAVAuthInfo	19

6.1.6.3	Simple data types and enumerations	21

6.1.6.3.1	Introduction	21

6.1.6.3.2	Simple data types	21

6.1.6.3.3	Enumeration: AuthResult	21

6.1.6.3.4	Enumeration: NotifType	21

6.1.7	Error Handling	22

6.1.7.1	General	22

6.1.7.2	Protocol Errors	22

6.1.7.3	Application Errors	22

6.1.8	Feature negotiation	22

6.1.9	Security	22

A.1	General	23

A.2	Nnef_Authentication API	23

A.3	<Service 2> API	26

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the stage 3 protocol and data model for the Nnef Service Based Interface. It provides stage 3 protocol definitions and message flows, and specifies the API for each service offered by the NEF(UAS-NF).

The 5G System stage 2 architecture and procedures are specified in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3].

The Technical Realization of the Service Based Architecture and the Principles and Guidelines for Services Definition are specified in 3GPP TS 29.500 [4] and 3GPP TS 29.501 [5].

The Uncrewed Aerial System Network Function (UAS-NF) provides the UAS-specific NEF services to NF service consumers (e.g. AMF, SMF). The UAS-NF is functionality within the NEF.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	3GPP TS 23.256: "Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking; Stage 2".

[7]	3GPP TR 21.900: "Technical Specification Group working methods".

[8]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[9]	OpenAPI Initiative, "OpenAPI 3.0.0 Specification", https://github.com/OAI/OpenAPI-Specification/blob/master/versions/3.0.0.md.

[10]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[11]	IETF RFC 7807: "Problem Details for HTTP APIs".

[12]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[13]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[14]	3GPP TS 29.510: "Network Function Repository Services; Stage 3".

[15]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

For the purposes of the present document, the terms and definitions given in 3GPP TS 23.256 [6] shall apply.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AA	Authorization/Authentication

BRID	Broadcast Remote Identification

BVLOS	Beyond Visual Line of Sight

C2	Command and Control

NRID	Networked Remote Identification

PEI	Permanent Equipment Identifier

RID	Remote Identification

TPAE	Third Party Authorized Entity

UAS	Uncrewed Aerial System

UAV	Uncrewed Aerial Vehicle

USS	UAS Service Supplier

UTM	Uncrewed Aerial System Traffic Management

UUAA	USS UAV AA

UUID	Universal Unique Identifier

# 4	Overview

## 4.1	Introduction

Within the 5GC, the NEF(UAS-NF) offers services to the AMF, SMF, and PCF via the Nnef service based interface, and the NEF(UAS-NF) offers services to the SMF+PGW-C via the Nnef service based interface for UUAA-SM and C2 procedure supported in EPS (see 3GPP TS 23.256 [6]).

Figure 4.1-1 provides the reference model (in service based interface representation and in reference point representation), with focus on the NEF(UAS-NF) and the scope of the present specification.

Figure 4.1-1: Reference model – NEF(UAS-NF)

The functionalities supported by the NEF(UAS-NF) are listed in clause 4.3.2 of 3GPP TS 23.256 [6].

# 5	Services offered by the NEF(UAS-NF)

## 5.1	Introduction

The table 5.1-1 shows the NEF(UAS-NF) Services and Service Operations:

Table 5.1-1 List of NEF(UAS-NF) Services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) | Mapped Service Operation |
| --- | --- | --- | --- | --- |
| Nnef_Authentication | AuthenticateAuthorize | Request/Response | AMF, SMF, SMF+PGW-C | Nnef_Authentication_AuthenticateAuthorize |
|  | Notification | Subscribe/Notify | AMF, SMF, SMF+PGW-C | Nnef_Authentication_Notification |

## 5.2	Nnef_Authentication Service

### 5.2.1	Service Description

The service allows communication of authentication and authorization messages between AMF/SMF and external AF (USS). An NF as service consumer (e.g. AMF, SMF, SMF+PGW-C) can authenticate or subscribe to receive notifications from NEF(UAS-NF) related to reauthentication, update authorization data or revoke authorization of the UAV.

### 5.2.2	Service Operations

#### 5.2.2.1	Introduction

The Nnef_Authentication service supports following service operations:

-	AuthenticateAuthorize

-	Notification

#### 5.2.2.2	AuthenticateAuthorize Service Operation

##### 5.2.2.2.1	General

The AuthenticateAuthorize service operation is used during the following procedure:

-	UUAA-MM and UUAA-SM procedures (see 3GPP TS 23.256 [6], clause 5.2.2 and clause 5.2.3, respectively)

-	C2 authorization (see 3GPP TS 23.256 [6], clause 5.2.5)

The Authenticate service operation is invoked by an NF Service Consumer (e.g. an AMF, SMF, SMF+PGW-C) towards the NEF(UAS-NF), when UUAA-MM is done at 5GS registration or UUAA-SM is done at PDU session establishment, respectively.

The NF Service Consumer (e.g. the AMF or the SMF or the SMF+PGW-C) shall send the authentication message to NEF(UAS-NF) by sending the HTTP POST request towards the UAV Authentications resource as shown in Figure 5.2.2.2.1-1.

Figure 5.2.2.2.1-1: AuthenticateAuthorize Service Operation

1.	The NF Service Consumer shall send a POST request to the resource representing the UAV Authentications resource of the NEF(UAS-NF) with a "UAVAuthInfo" object in the request body, including:

-	gpsi IE set to GPSI (in the format of External Identifier) of the UAV;

-	serviceLevelId IE set to Service Level Device Identity;

-	authMsg IE contains the service-level AA message;

-	authNotificationURI IE provides the notification URI to receive notifications related to authentication;

-	authServerAddress IE provides the Authorization Server Address, e.g. Authorization Server FQDN. This IE is not included for intermediate round-trip authentication messages;

-	nfType IE carries the NF type of the  NF service consumer (e.g. AMF or SMF or SMF+PGW-C); and

-	userLocInfo IE provides the user location information (e.g. cell Id).

If the NF Service Consumer is SMF or SMF+PGW-C, the "UAVAuthInfo" also may include:

-	ipAddr IE carries the IP Address associated with the PDU session;

-	pei IE carries the PEI;

-	dnn IE carries the DNN which can be used by NEF(UAS-NF) later on to create a subscription at SMF; and

-	sNssai IE carries the S-NSSAI which can be used by NEF(UAS-NF) later on to create a subscription at SMF.

2a.	On success, the NEF(UAS-NF) shall store the result and return "200 OK".

For intermediate round-trip messages, the payload body (i.e. UAVAuthResponse) shall contain the GPSI of the UAV and Service Level Device Identity. The payload body optionally includes "authMsg".

For the final NEF(UAS-NF) to NF service consumer message, the payload body (i.e. UAVAuthResponse) shall contain the GPSI of the UAV, authSessCorrId attribute and "authResult" attribute. If the UAV is authenticated successfully, the NEF(UAS-NF) shall set the "authResult" attribute to "AUTH_SUCCESS". The payload body optionally contains the authorized Service Level Device Identity and "authMsg" payload delivering configuration information to the UAV. The AMF forwards the message transparently to UE over NAS MM.

2b.	On failure or redirection, one of the HTTP status codes listed in Table 6.1.7.3-1 shall be returned. For a 4xx/5xx response, the message body containing a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in Table 6.1.7.3-1.

If the NEF(UAS NF) cannot successfully fulfil the received HTTP POST request due to an internal error or an error in the HTTP POST request, the NEF(UAS NF) shall send the HTTP error response as specified in clause 6.1.7.

If the UAV authentication is failed, the NEF(UAS NF) shall reject the request with an HTTP "403 Forbidden" response message including the "cause" attribute of the ProblemDetails data structure set to "AUTHENTICATION_FAILURE". NEF(UAS-NF) may also include an indication of uasResourceRelease if received from the USS.

In above steps, while there is no expected response from the USS in the case of time out, the NEF(UAS-NF) shall return HTTP status code "504 Gateway Timeout", with the message body containing a ProblemDetails structure with the "cause" attribute set to "PEER_NOT_RESPONDING".

#### 5.2.2.3	AuthNotify Service Operation

##### 5.2.2.3.1	General

The AuthNotify service operation is used during the following procedure:

-	USS Initiated reauthentication (see 3GPP TS 23.256 [6], clause 5.2.4)

-	USS Initiated update authorization data or revoke authorization of the UAV

The AuthNotify service operation is invoked by the NEF(UAS-NF) to inform a NF Service Consumer (e.g. AMF, SMF, SMF+PGW-C), when USS triggers reauthentication, update authorization data or revoke authorization of the UAV. NEF(UAS-NF) shall determine the NF service consumer based on the previously stored UUAA context during the successful UUAA procedure as defined in clause 5.2.2.2.

The NEF(UAS-NF) shall send the AuthNotify request by sending the HTTP POST method towards the Notification URI as shown in Figure 5.2.2.3.1-1.

Figure 5.2.2.3.1-1: AuthNotify Service Operation

1.	The NEF(UAS-NF) shall send a POST request towards the Notification URI received in the Authenticate service operation request (See clause 5.2.2.2.1). The NEF(UAS-NF) shall be able to determine the NF type of the NF service consumer by nfType IE received in the Authenticate service operation request. The request body shall contain a "AuthNotification" object containing the reauthentication information or update authorization information or revoke authorization indication.

When the procedure is used for reauthentication or reauthorization/update authorization information, the AuthNotification object includes:

- the gpsi IE set to the GPSI (in the format of External Identifier) of the given UAV required to be reauthenticated;

- serviceLevelId IE set to the Service Level Device Identity of the UAV;

- authMsg IE contains the service-level AA message;

- ipAddr IE set to the IP Address associated with the PDU session if the NF service consumer is SMF or SMF+PGW-C; and

- authSessCorrId IE set to the authentication session correlation id;

- notifType IE set to REAUTH used for reauthentication and/or notifType IE set to UPDATEAUTH used for update authorization data.

Editor's NOTEs:	Notification type is yet to be defined in SA2

When the procedure is used for authorization revocation, the AuthNotification object includes:

- the gpsi IE set to the GPSI (in the format of External Identifier) of the given UAV;

- serviceLevelId IE set to the Service Level Device Identity of the UAV;

- authSessCorrId IE set to the authentication session correlation id;

- ipAddr IE set to the IP Address associated with the PDU session if the NF service consumer is SMF or SMF+PGW-C;

- causeOfRevocation IE set to cause of revocation received from USS; and

- notifType IE set to REVOKE.

2a.	On success, "204 No content" shall be returned without response body.

2b.	On failure or redirection, one of the HTTP status code listed in Table 6.1.5.2.3.1-3 shall be returned. For a 4xx/5xx response, the response body should contain a "ProblemDetails" object.

If the NF service consumer cannot successfully fulfil the received HTTP POST request due to an internal error or an error in the HTTP POST request, the NF service consumer shall send an HTTP error response as specified in clause 6.1.7.

# 6	API Definitions

## 6.1	Nnef_Authentication Service API

### 6.1.1	Introduction

The Nnef_Authentication shall use the Nnef_Authentication API.

The API URI of the Nnef_Authentication API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests from the NF service consumer towards the NF service producer shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [5].

-	The <apiName> shall be "nnef-authentication".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 6.1.3.

### 6.1.2	Usage of HTTP

#### 6.1.2.1	General

HTTP/2, IETF RFC 7540 [8], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

The OpenAPI [9] specification of HTTP messages and content bodies for the Nnef_Authentication API is contained in Annex A.

#### 6.1.2.2	HTTP standard headers

##### 6.1.2.2.1	General

See clause 5.2.2 of 3GPP TS 29.500 [4] for the usage of HTTP standard headers.

##### 6.1.2.2.2	Content type

JSON, IETF RFC 8259 [10], shall be used as content type of the HTTP bodies specified in the present specification as specified in clause 5.4 of 3GPP TS 29.500 [4]. The use of the JSON format shall be signalled by the content type "application/json".

"Problem Details" JSON object shall be used to indicate additional details of the error in a HTTP response body and shall be signalled by the content type "application/problem+json", as defined in IETF RFC 7807 [11].

#### 6.1.2.3	HTTP custom headers

The mandatory HTTP custom header fields specified in clause 5.2.3.2 of 3GPP TS 29.500 [4] shall be applicable.

### 6.1.3	Resources

#### 6.1.3.1	Overview

The structure of the Resource URIs of the Nnef_Authentication API is shown in Figure 6.1.3.1-1

Figure 6.1.3.1-1: Resource URI structure of the Nnef_Authentication API

Table 6.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| uav-authentications | /uav-authentications | POST | A UAV authentication |

#### 6.1.3.2	Resource: uav-authentications

The resource represents UAV Authentications to be done with the NEF(UAS-NF).

##### 6.1.3.2.1	Description

#### 6.1.3.2.2	Resource Definition

Resource URI: {apiRoot}/nnef-authentication/<apiVersion>/uav-authentications

This resource shall support the resource URI variables defined in table 6.1.3.2.2-1.

Table 6.1.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |
| apiVersion | string | See clause 6.1.1 |

##### 6.1.3.2.3	Resource Standard Methods

###### 6.1.3.2.3.1	POST

This method performs UAV authentication.

This method shall support the URI query parameters specified in table 6.1.3.2.3.1-1.

Table 6.1.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.3.2.3.1-3.

Table 6.1.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| UAVAuthInfo | M | 1 | Represents the data to be used for UAV authentication |

Table 6.1.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| UAVAuthResponse | M | 1 | 200 OK | <Meaning of the success case> or <Meaning of the error case with additional statement regarding error handling> |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing a different URI, or the same URI if a request is redirected to the same target resource via a different SCP. In the former case, the URI shall be an alternative URI of the resource located on an alternative service instance within the same NEF(UAS-NF) or NEF(UAS-NF) (service) set. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing a different URI, or the same URI if a request is redirected to the same target resource via a different SCP. In the former case, the URI shall be an alternative URI of the resource located on an alternative service instance within the same NEF(UAS-NF) or NEF(UAS-NF) (service) set. (NOTE 2) |
| UAVAuthFailure | O | 0..1 | 403 Forbidden | This represents that the UAV authentication is failed, the "cause" attribute of the ProblemDetails data structure set to: - AUTHENTICATION_FAILURE See table 6.1.7.3-1 for the description of these errors. |
| ProblemDetails | O | 0..1 | 504 Gateway Timeout | The "cause" attribute may be used to indicate the following application error: - PEER_NOT_RESPONDING See table 6.1.7.3-1 for the description of the error. |
| NOTE 1: The mandatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.1.3.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same NEF(UAS-NF) or NEF(UAS-NF) (service) set. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

Table 6.1.3.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same NEF(UAS-NF) or UAS-NF/NEF (service) set. Or the same URI, if a request is redirected to the same target resource via a different SCP. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the request is redirected |

##### 6.1.3.2.4	Resource Custom Operations

None

### 6.1.5	Notifications

#### 6.1.5.1	General

This clause specifies the notifications provided by the Nnef_Authentication service.

Notifications shall comply to clause 6.2 of 3GPP TS 29.500 [4] and clause 4.6.2.3 of 3GPP TS 29.501 [5].

#### 6.1.5.2	Authentication Notification

##### 6.1.5.2.1	Description

The NF Service Consumer (e.g. the AMF or SMF or SMF+PGW-C) provides the Notification URI for getting notified about reauthentication requested by the USS. The NEF(UAS-NF) shall notify the NF Service Consumer when reauthentication is requested by the USS.

##### 6.1.5.2.2	Target URI

The Notification URI "{notifUri}" shall be used with the callback URI variables defined in table 6.1.5.2.2-1.

Table 6.1.5.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notificationUri | String formatted as URI with the Callback Uri |

##### 6.1.5.2.3	Standard Methods

###### 6.1.5.2.3.1	POST

This method shall support the request data structures specified in table 6.1.5.2.3.1-1 and the response data structures and response codes specified in table 6.1.5.2.3.1-2.

Table 6.1.5.2.3.1-1: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AuthNotification | M | 1 | Contains the reauthentication information. |

Table 6.1.5.2.3.1-2: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful notification of reauthentication |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing a different URI. The URI shall be an alternative URI of the resource located on an alternative service instance within the same NF consumer where the notification should be sent. If an SCP redirects the message to another SCP then the location header field shall contain the same URI or a different URI pointing to the endpoint of the NF service consumer to which the notification should be sent. (NOTE 2) |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing a different URI. The URI shall be an alternative URI of the resource located on an alternative service instance within the same NF consumer where the notification should be sent. If an SCP redirects the message to another SCP then the location header field shall contain the same URI or a different URI pointing to the endpoint of the NF service consumer to which the notification should be sent. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply, with response body containing an object of ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). NOTE 2: RedirectResponse may be inserted by an SCP, see clause 6.10.9.1 of 3GPP TS 29.500 [4]. |  |  |  |  |

Table 6.1.5.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | A URI pointing to the endpoint of NF service consumer to which the notification should be sent |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the notification is redirected |

Table 6.1.5.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | A URI pointing to the endpoint of NF service consumer to which the notification should be sent |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance ID towards which the notification is redirected |

### 6.1.6	Data Model

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nnef_Authentication service based interface protocol.

Table 6.1.6.1-1: Nnef_Authentication specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| UAVAuthInfo | 6.1.6.2.2 | Information within Authenticate Request |  |
| AuthNotification | 6.1.6.2.3 | Information within notification |  |
| UAVAuthResponse | 6.1.6.2.4 | Information within Authenticate Response |  |
| UAVAuthFailure | 6.1.6.2.5 | Information within Authenticate Response |  |
| AuthResult | 6.1.6.3.3 | Enumeration indicating authentication result |  |
| NotifType | 6.1.6.3.4 | Enumeration Notification type |  |

Table 6.1.6.1-2 specifies data types re-used by the Nnef_Authentication service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nnef_Authentication service based interface.

Table 6.1.6.1-2: Nnef_Authentication re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| IpAddr | 3GPP TS 29.571 [15] | IP address |  |
| Pei | 3GPP TS 29.571 [15] | Permanent Equipment Identifier |  |
| Uri | 3GPP TS 29.571 [15] | Uri |  |
| Gpsi | 3GPP TS 29.571 [15] | GPSI |  |
| ExtSnssai | 3GPP TS 29.571 [15] | Ext Snssai |  |
| Dnn | 3GPP TS 29.571 [15] | DNN information |  |
| UserLocation | 3GPP TS 29.571 [15] | User location |  |
| RedirectResponse | 3GPP TS 29.571 [15] | Contains redirection related information |  |
| NFType | 3GPP TS 29.510 [14] | NF Type |  |
| RefToBinaryData | 3GPP TS 29.571 [15] | authMsg data |  |

#### 6.1.6.2	Structured data types

##### 6.1.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 6.1.6.2.2	Type: UAVAuthInfo

Table 6.1.6.2.2-1: Definition of type UAVAuthInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| gpsi | Gpsi | M | 1 | GPSI of the UAV |  |
| serviceLevelId | string | M | 1 | Service Level Device Identity of the UAV |  |
| ipAddr | IpAddr | O | 0..1 | This IE may be present if the NF Service Consumer is the SMF or SMF+PGW-C. When present, this IE indicates the IP address associated with the PDU session. |  |
| authMsg | RefToBinaryData | O | 0..1 | Contains the service-level AA message. |  |
| pei | Pei | O | 0..1 | This IE may be present if the NF Service Consumer is the SMF or SMF+PGW-C. When present, PEI associated with the UAV. |  |
| authServerAddress | string | O | 0..1 | Provides the Authorization Server Address, e.g. Authorization Server FQDN. |  |
| authNotificationURI | Uri | C | 0..1 | This IE shall be present in the initial authentication message or SMF+PGW-C. It carries the notification URI to receive authentication related notifications |  |
| dnn | Dnn | C | 0..1 | This IE may be present if the NF Service Consumer is the SMF or SMF+PGW-C. When present, this IE indicates DNN associated with the PDU session. |  |
| sNssai | ExtSnssai | C | 0..1 | This IE may be present if the NF Service Consumer is the SMF. When present, this IE indicates the S-NSSAI associated with the PDU session. |  |
| ueLocInfo | UserLocation | O | 0..1 | This IE shall contain the UE location information if it is available. |  |
| nfType | NFType | M | 0..1 | NFType of the NF service consumer. Possible NFType values supported in this release of the specification are - AMF - SMF |  |

##### 6.1.6.2.3	Type: AuthNotification

Table 6.1.6.2.3-1: Definition of type AuthNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| gpsi | Gpsi | M | 1 | GPSI of the UAV |  |
| serviceLevelId | string | M | 1 | Service Level Device Identity of the UAV |  |
| authMsg | RefToBinaryData | O | 0..1 | Contains the service-level AA message. |  |
| ipAddr | IpAdd | O | 0..1 | When present, this IE indicates the IP address associated with the PDU session. This IE may be present if the NF Service Consumer is the SMF or SMF+PGW-C. |  |
| authSessCorrId | string | O | 0..1 | When present, this IE shall contain the authentication Session Correlation Id. |  |
| notifType | NotifType | M | 1 | This IE shall contain the notification type. |  |
| revokeCause | string | C | 0..1 | This IE shall contain the cause of revocation. This IE may be present if the NnotifType is set to "REVOKE". |  |

##### 6.1.6.2.4	Type: UAVAuthResponse

Table 6.1.6.2.4-1: Definition of type UAVAuthResponse

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| gpsi | Gpsi | M | 1 | GPSI of the UAV |  |
| authResult | AuthResult | C | 0..1 | This IE shall be present for the final NEF(UAS-NF) to NF service consumer message. Conveys the UAV authentication result |  |
| authMsg | RefToBinaryData | O | 0..1 | Contains the service-level AA message. |  |
| serviceLevelId | string | O | 0..1 | This IE contains the authorized Service Level Device Identity |  |
| authSessCorrId | string | O | 0..1 | This IE shall contain the authentication Session Correlation Id. |  |

##### 6.1.6.2.5	Type: UAVAuthFailure

Table 6.1.6.2.x-1: Definition of type UAVAuthFailure

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| error | ProblemDetails | M | 0..1 | Represents the application error information. The application level error cause shall be encoded in the "cause" attribute. |  |
| uasResourceRelease | boolean | O | 0..1 | This IE may be present and set to "true" when the NEF(UAS NF) reject the request with an HTTP "403 Forbidden" response message including the "cause" attribute of the ProblemDetails data structure set to "AUTHENTICATION_FAILURE" to indicate the UAS resource release is requested. |  |

#### 6.1.6.3	Simple data types and enumerations

##### 6.1.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.1.6.3.2	Simple data types

The simple data types defined in table 6.1.6.3.2-1 shall be supported.

Table 6.1.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 6.1.6.3.3	Enumeration: AuthResult

The enumeration AuthResult represents the result of authentication and/or authorization. It shall comply with the provisions defined in table 6.1.5.3.3-1.

Table 6.1.6.3.3-1: Enumeration AuthResult

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "AUTH_SUCCESS" | The UAV authentication has succeeded. |  |

##### 6.1.6.3.4	Enumeration: NotifType

The enumeration NotifType represents the type of notification. It shall comply with the provisions defined in table 6.1.6.3.4-1.

Table 6.1.6.3.4-1: Enumeration NotifType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "REAUTH" | The UAV needs to be reauthenticated. |  |
| "UPDATEAUTH" | Authorization data needs to be updated to UAV. |  |
| "REVOKE " | Revoke UAV authentication and authorization |  |

### 6.1.7	Error Handling

#### 6.1.7.1	General

For the Nnef_Authentication API, HTTP error responses shall be supported as specified in clause 4.8 of 3GPP TS 29.501 [5]. Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [4] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [4].

In addition, the requirements in the following clauses are applicable for the Nnef_Authentication API.

#### 6.1.7.2	Protocol Errors

No specific procedures for the Nnef_Authentication service are specified.

#### 6.1.7.3	Application Errors

The application errors defined for the Nnef_Authentication service are listed in Table 6.1.7.3-1.

Table 6.1.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| AUTHENTICATION_FAILURE | 403 Forbidden | The UAV authentication is failed |
| PEER_NOT_RESPONDING | 504 Gateway Timeout | No response is received from the remote peer (i.e. USS) when time out. |

## 6.1.8	Feature negotiation

The optional features in table 6.1.8-1 are defined for the Nnef_Authentication API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 6.1.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

## 6.1.9	Security

As indicated in 3GPP TS 33.501 [12] and 3GPP TS 29.500 [4], the access to the Nnef_Authentication API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [13]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [14]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming services offered by the Nnef_Authentication API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [14], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Nnef_Authentication service.

The Nnef_Authentication API defines a single scope "nnef-authentication" for the entire service, and it does not define any additional scopes at resource or operation level.

Annex A (normative):
OpenAPI specification

## A.1	General

This Annex specifies the formal definition of the API(s) defined in the present specification. It consists of OpenAPI 3.0.0 specifications in YAML format, following guidelines in 3GPP TS 29.501 [5].

This Annex takes precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API(s).

NOTE 1:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification files contained in this 3GPP Technical Specification are available on a Git-based repository that uses the GitLab software version control system (see 3GPP TS 29.501 [5] clause 5.3.1 and 3GPP TR 21.900 [7] clause 5B).

## A.2	Nnef_Authentication API

openapi: 3.0.0

info:

title: Nnef_Authentication

version: 1.0.0-alpha.1

description: |

NEF Auth Service.

© 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.256 V0.3.0; 5G System;Uncrewed Aerial Systems Network Function (UAS-NF);Aerial Management Services; Stage 3

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.256/

servers:

- url: '{apiRoot}/nnef-authentication/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- nnef-authentication

paths:

/uav-authentications:

post:

summary:  UAV authentication

tags:

- UAV authentication

requestBody:

description: UAV authentication

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/UAVAuthInfo'

responses:

'200':

description: UAV Auth response or message exchange

content:

application/json:

schema:

$ref: '#/components/schemas/UAVAuthResponse'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

authNotification:

'{request.body#/authNotification }':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AuthNotification'

responses:

'204':

description: Successful Notification response

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nnef-authentication: Access to the Nnef_authentication API

schemas:

#

# STRUCTURED DATA TYPES

#

UAVAuthInfo:

description: UAV auth data

type: object

required:

- gpsi

- serviceLevelId

- nfType

properties:

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

serviceLevelId:

type: string

authNotificationURI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

ipAddr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/IpAddr'

pei:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

authServerAddress:

type: string

authMsg:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

ueLocInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

sNssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ExtSnssai'

nfType:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/NFType'

UAVAuthResponse:

description: UAV auth response data

type: object

required:

- gpsi

properties:

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

serviceLevelId:

type: string

authSessCorrId:

type: string

authMsg:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

authResult:

$ref: '#/components/schemas/AuthResult'

uasResourceRelease:

type: boolean

default: false

AuthNotification:

description: UAV related notification

type: object

required:

- gpsi

- serviceLevelId

- notifType

properties:

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

serviceLevelId:

type: string

authSessCorrId:

type: string

authMsg:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

notifType:

$ref: '#/components/schemas/NotifType'

revokeCause:

type: string

ipAddr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/IpAddr'

UAVAuthFailure:

description: UAV auth failure

type: object

required:

- error

properties:

error:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

uasResourceRelease:

type: boolean

#

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

AuthResult:

description: Enumeration representing the result of authentication and/or authorization.

anyOf:

- type: string

enum:

- AUTH_SUCCESS

- type: string

NotifType:

description: Enumeration representing the type of notification.

anyOf:

- type: string

enum:

- REAUTH

- UPDATEAUTH

- REVOKE

- type: string

## A.3	<Service 2> API

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-04 | CT4#103-e | C4-212292 |  |  |  | Initial TS skeleton. | 0.0.0 |
| 2021-04 | CT4#103-e | C4-212598 |  |  |  | Added scope, introduction, references, and abbreviations. | 0.1.0 |
| 2021-05 | CT4#104-e | C4-213529 |  |  |  | Aligned introduction terminology. Added, updated references. | 0.2.0 |
| 2021-08 | CT4#105-e | C4-214758 |  |  |  | Aligned interfaces, added Nnef_Auth service. | 0.3.0 |
| 2021-10 | CT4#106-e | C4-215522 |  |  |  | Several alignments with stage-2, clean-up, added OpenAPI for Nnef_Auth. | 0.4.0 |
| 2021-11 | CT4#107-e | C4-216474 |  |  |  | Several further alignments with stage-2: aligned API and operation names, terminology. Further clean-up done. | 0.5.0 |