| 3GPP TS 29.392 V1.0.0 (2025-09) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Application layer support for MMTel; MMTel Enabler Server Services; Stage 3; (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	8

4	Overview	8

5	Services offered by the MMTel Enabler Server	10

5.1	Introduction	10

5.2	MMTel_DCAppManagement Service	10

5.2.1	Service Description	10

5.2.2	Service Operations	10

5.2.2.1	Introduction	11

5.2.2.2	MMTel_DCAppManagement_Configuration	11

5.2.2.2.1	General	11

5.2.2.2.2	DC application and profile configuration	11

5.2.2.2.3	<Procedure 2 using service operation 1 of service 1>	12

5.2.2.3	MMTel_DCAppManagement_Update	12

5.2.2.3.1	General	12

5.2.2.3.2	DC application and profile update	12

5.2.2.4	MMTel_DCAppManagement_Delete	12

5.2.2.4.1	General	12

5.2.2.4.2	DC application delete	12

5.2.2.5	MMTel_DCAppManagement_Retrieval	13

5.2.2.5.1	General	13

5.2.2.5.2	DC application profile information retrieval	13

5.3	MMTel_DCAppCall Service	14

5.4	MMTel_CallControlHandling Service	15

5.5	<Service 4> Service	16

6	API Definitions	17

6.1	MMTel_DCAppManagement API	17

6.1.1	Introduction	17

6.1.2	Usage of HTTP and common API related aspects	17

6.1.3	Resources	17

6.1.3.1	Overview	17

6.1.3.2	Resource: DC APP	18

6.1.3.2.1	Description	18

6.1.3.2.2	Resource Definition	18

6.1.3.2.3	Resource Standard Methods	19

6.1.3.2.4	Resource Custom Operations	20

6.1.3.2.4.1	Overview	20

6.1.3.2.4.2	Operation: Configure	20

6.1.3.2.4.3	Operation: Update	21

6.1.3.2.4.4	Operation: Delete	21

6.1.4	Custom Operations without associated resources	22

6.1.5	Notifications	22

6.1.6	Data Model	22

6.1.6.1	General	22

6.1.6.2	Structured data types	23

6.1.6.2.1	Introduction	23

6.1.6.2.2	Type: DcAppConfigReq	24

6.1.6.2.3	Type: DcAppConfigParameters	25

6.1.6.2.4	Type: DcAppConfigResp	26

6.1.6.2.5	Type: DcAppConfigResponseParameters	26

6.1.6.2.6	Type: DcAppUpdateReq	26

6.1.6.2.7	Type: DcAppUpdateParameters	27

6.1.6.2.8	Type: DcAppStatResp	28

6.1.6.2.9	Type: DcAppResponseParameters	28

6.1.6.2.10	Type: DcAppIdReq	28

6.1.6.2.11	Type: DcAppIdResp	28

6.1.6.2.12	Type: Condition	29

6.1.6.3	Simple data types and enumerations	29

6.1.6.3.1	Introduction	29

6.1.6.3.2	Simple data types	29

6.1.6.3.3	Enumeration: AppLoadPhase	29

6.1.6.3.4	Enumeration: SupportScenario	29

6.1.6.3.5	Enumeration:  Status	29

6.1.7	Error Handling	30

6.1.7.1	General	30

6.1.7.2	Protocol Errors	30

6.1.7.3	Application Errors	30

6.1.8	Feature negotiation	30

6.1.9	Security	30

6.2	MMTel_DCAppCall API	31

6.3	MMTel_CallControlHandling API	32

6.4	<Service 4> API	33

7	Using Common API Framework	34

7.1	General	34

7.2	Security	34

Annex A (normative): OpenAPI specification	35

A.1	General	35

A.2	MMTel_DCAppManagement API	36

A.3	MMTel_DCAppCall API	40

A.4	MMTel_CallControlHandling API	41

A.5	<Service 4> API	42

Annex <B> (informative): Withdrawn API versions	43

B.1	General	43

B.2	<Service 1> API	43

B.3	<Service 2> API	43

Annex <X> (informative): Change history	44

For definitive guidance on drafting 3GPP TSs and TRs, see 3GPP TS 21.801.

Ensure all blue guidance text is removed before submitting the TS/TR to the TSG for approval.

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the stage 3 Protocol and data model for the MMTel Enabler Server services, enabling the support of MMTel applications and services over 3GPP networks. It provides stage 3 protocol definitions and message flows, and specifies the API of each service  offered by the MMTel Enabler Server over the MMTel-2/3 interface. The stage 2 application layer architecture, functional requirements, procedures and information flows necessary for MMTel Service are contained in 3GPP TS 23.392 [2].

The stage 2 application layer architecture for MMTel, functional requirements, procedures and information flows necessary for enabling MMTel applications over 3GPP networks are specified in 3GPP TS 23.392 [6].

The common protocol and interface aspects for API definition are specified in clause 5.2 of 3GPP TS 29.122 [2].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 29.122: "T8 reference point for Northbound Application Programming Interfaces (APIs)".

[3]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[4]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[5]	3GPP TR 21.900: "Technical Specification Group working methods".

[6]	3GPP TS 23.392: "Application enablement aspects for MMTel".

[7]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[8]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs; Stage 2".

[9]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs; Stage 3".

[10]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs".

[11]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

For the purpose of the present document, the terms and definitions given in clause 3 of 3GPP TS 23.392 [6] also apply, including the ones referencing other specifications.

## 3.2	Symbols

Void

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

# 4	Overview

3GPP TS 23.392 [2] has specified the application layer architecture, architectural requirements, procedures, information flows, in order to support the Application enablement for MMTel Service, mainly including DC Application management, DC Application downloading control, MMTel service usage and Multiple call control handling.

The present document specifies the APIs needed to support MMTel Services for interworking between the MMTel Enabler Server and the Controlling Application Server or Application Server, including the following functionalities:

1.	Server-side functionality with the DC application configuration, update, deletion and information query, provided by the MMTel Enabler Server over the MMTel-2 interface.

2.	Server-side functionality to provide capabilities to application providers/Vertical service providers to use MMTel services over the MMTel-3 interface.

Figure 4-1 shows the reference model of the MMTel Application Enabler Layer, with a focus on the MMTel Enabler Server:

Figure 4-1: MMTel Application Enabler Layer functional model

# 5	Services offered by the MMTel Enabler Server

## 5.1	Introduction

Table 5.1-1 summarizes the corresponding APIs defined for this specification.

Table 5.1-x: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | API Name | Annex |
| --- | --- | --- | --- | --- | --- |
| <service name> | <ref clause> | <short description as included in the OpenAPI file> | <file name> | <apiName in the URI> | <ref Annex> |

NOTE:	When 3GPP TS 29.122 [2] is referenced for the common protocol and interface aspects for API definition in this document, the MMTel Server) takes the role of the SCEF and the service consumer takes the role of the SCS/AS.

## 5.2	MMTel_DCAppManagement Service

### 5.2.1	Service Description

The MMTel_DCAppManagement Service as defined in clause 8.2 in 3GPP TS 23.392 [2], is provided by the MMTel Enabler Server.

This service:

-	allows Controlling Application Server invokes services provided by a MMTel Enabler Server to config DC application and profile to the MMTel Enabler Server;

-	allows Controlling Application Server invokes services provided by a MMTel Enabler Server to update DC application and profile to the MMTel Enabler Server.

-	allows Controlling Application Server invokes services provided by a MMTel Enabler Server to delete DC application.

-	allows Controlling Application Server invokes services provided by a MMTel Enabler Server to obtain detail DC application profile information on the MMTel Enabler Server.

### 5.2.2	Service Operations

#### 5.2.2.1	Introduction

The service operations defined for MMTel_DCAppManagement Service is shown in the table 5.2.2.1-1

Table 5.2.2.1-1: Operations of the MMTel_DCAppManagement Service

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| MMTel_DCAppManagement_Configure | This service operation is used by the service consumer to configure DC application and profile to the MMTel Enabler Server. | e.g., Controlling Application Server |
| MMTel_DCAppManagement_Update | This service operation is used by Controlling Application Server to update an existing DC application profile information or DC application. | e.g., Controlling Application Server |
| MMTel_DCAppManagement_Delete | This service operation is used by the service consumer to delete an existing DC application. | e.g., Controlling Application Server |
| MMTel_DCAppManagement_ Retrieval | This service operation is used by the service consumer to retrieve the details of an existing DC application profile information. | e.g., Controlling Application Server |

#### 5.2.2.2	MMTel_DCAppManagement_Configure

##### 5.2.2.2.1	General

This service operation is used by the service consumer to configure DC application and profile to the MMTel Enabler Server.

The following procedures are supported by the "MMTel_DCAppManagement_Configure" service operation:

-	DC Application and Profile Configuration.

##### 5.2.2.2.2	DC Application and Profile Configuration

Figure 5.2.2.2.2-1 depicts a scenario where a Controlling Application Server sends a request to the MMTel Enabler Server to configure DC application and profile (see also clause 8.2.2 of 3GPP°TS°23.392°[6]).

Figure 5.2.2.2.2-1: Procedure for DC Application and Profile Configuration

1.	In order to configure DC application and profile, the Controlling Application Server shall send an HTTP POST request (i.e., custom operation "Configure") to the MMTel Enabler Server targeting the "DC APP" collection resource, with the request body including the DcAppConfigReq data structure.

2a.	Upon success, the MMTel Enabler Server shall respond with an HTTP "201 Created" status code with the response body containing a representation of the created "DC APP" resource and potentially additional information within the DcAppConfigResp data structure.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.1.6.

#### 5.2.2.3	MMTel_DCAppManagement_Update

##### 5.2.2.3.1	General

This service operation is used by Controlling Application Server to update an existing DC application profile information or DC application at the MMTel Enabler Server.

The following procedures are supported by the "MMTel_DCAppManagement_Configure" service operation:

-	DC Application and Profile Update.

##### 5.2.2.3.2	DC Application and Profile Update

Figure 5.2.2.3.2-1 depicts a scenario where a Controlling Application Server sends a request to the MMTel Enabler Server to update existing DC application and profile (see also clause 8.2.3 of 3GPP°TS°23.392°[6]).

Figure 5.2.2.3.2-1: Procedure for DC Application and Profile Update

1.	In order to update existing DC application or DC application profile related information, the Controlling Application Server shall send an HTTP POST request (i.e., custom operation "Update") to the MMTel Enabler Server targeting the corresponding "DC APP" resource, with the request body including the DcAppUpdateReq data structure.

2a.	Upon success, the MMTel Enabler Server shall respond with an HTTP "200 OK" status code with the response body containing a representation of the updated "DC APP" resource within the DcAppStatResp data structure.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.1.6.

#### 5.2.2.4	MMTel_DCAppManagement_Delete

##### 5.2.2.4.1	General

This service operation is used by the service consumer to delete an existing DC application at the MMTel Enabler Server.

The following procedures are supported by the "MMTel_DCAppManagement_Configure" service operation:

-	DC Application and Profile Deletion.

##### 5.2.2.4.2	DC Application and Profile Deletion

Figure 5.2.2.4.2-1 depicts a scenario where a Controlling Application Server sends a request to the MMTel Enabler Server to delete existing DC application and profile (see also clause 8.2.4 of 3GPP°TS°23.392°[6]).

Figure 5.2.2.4.2-1: Procedure for DC Application and Profile Deletion

1.	In order to configure DC application and profile, the Controlling Application Server shall send an HTTP POST request (i.e., custom operation "Delete") to the MMTel Enabler Server targeting the "DC APP" collection resource, with the request body including the DcAppIdReq data structure;

2.	Upon success, the MMTel Enabler Server shall respond with an HTTP "200 OK" status code with the response body containing a representation of the updated "DC APP" resource within the DcAppStatResp data structure.

3.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.1.6.

#### 5.2.2.5	MMTel_DCAppManagement_Retrieval

##### 5.2.2.5.1	General

This service operation is used by the service consumer to retrieve the details of an existing DC application profile information at the MMTel Enabler Server.

The following procedures are supported by the "MMTel_DCAppManagement_Configure" service operation:

-	DC Application and Profile Retrieval.

##### 5.2.2.5.2	DC Application and Profile Retrieval

Figure 5.2.2.5.2-1 depicts a scenario where a Controlling Application Server sends a request to the MMTel Enabler Server to retrieve DC application profile related information (see also clause 8.2.5 of 3GPP°TS°23.392°[6]).

Figure 5.2.2.5.2-1: Procedure for DC Application and Profile Retrieval

1.	In order to retrieve existing DC application profile related information, the Controlling Application Server shall send an HTTP GET request to the MMTel Enabler Server targeting the "DC APP" collection resource, with the request body including the DcAppIdReq data structure.

2.	Upon success, the MMTel Enabler Server shall respond with an HTTP "200 OK" status code with the response body containing a representation of the updated "DC APP" resource within the DcAppIdResp data structure.

3.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP GET response body, as specified in clause 6.1.6.

## 5.3	MMTel_DCAppCall Service

### 5.3.1	Service Description

The MMTel_ DCAppCall Service as defined in clause 8.4 in 3GPP TS 23.392 [2], is provided by the MMTel Enabler Server.

This service:

-	allows Application Server invokes the service provided by a MMTel Enabler Server to establish a Third-Party Call with Data Channel capability;

-	allows Application Server invokes services provided by a MMTel Enabler Server to add an A2P Data Channel to an existing IMS session for establishment of an Application Call with Data Channel capability.

-	allows the MMTel Enabler Server to notify to the Application Server about DC resource information.

### 5.3.2	Service Operations

#### 5.3.2.1	Introduction

The service operation defined for MMTel_DCAppCall API is shown in the table 5.3.2.1-1.

Table 5.3.2.1-1: Operations of the MMTel_DCAppCall API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| MMTel_DCAppCall_DCThirdPartyCallReq | The service operation is used by the service consumer to request to establish a Third-Party Call with Data Channel capability | Application Server |
| MMTel_DCAppCall_UpdateDCMedia | The service operation is used by the service consumer to request to add an A2P Data Channel media to an existing IMS session. | Application Server |
| MMTel_DCAppCall_Notify | The service operation is used by the MMTel Enabler Server to notify the service consumer about any update of DC resource information. | MMTel Enabler Server |

## 5.4	MMTel_CallControlHandling Service

And so on if there are more than two services offered by the MMTel Enabler Server. Same structure as in clause 5.2.

## 5.5	<Service 4> Service

And so on if there are more than two services offered by the MMTel Enabler Server. Same structure as in clause 5.2.

# 6	API Definitions

## 6.1	MMTel_DCAppManagement API

### 6.1.1	Introduction

The MMTel_DCAppManagement shall use the MMTel_DCAppManagement API.

The API URI of theMMTel_DCAppManagement shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 5.2.4 of 3GPP TS 29.122 [2], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [2].

-	The <apiName> shall be "mmtel-dcappmgmt".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [2].

### 6.1.2	Usage of HTTP and common API related aspects

The provisions of clause 5.2 of 3GPP TS 29.122 [2] shall apply for the MMTel_DCAppManagement API.

### 6.1.3	Resources

#### 6.1.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.1.3.1-1 depicts the resource URIs structure for the MMTel_DCAppManagement API.

Figure 6.1.3.1-1: Resource URI structure of the MMTel_DCAppManagement API

Table 6.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| DC APP | /dcapps/configure | POST | Configure DC application and DC application profile. |
|  | /dcapps/update | POST | Update existing DC application and DC application profile. |
|  | /dcapps/delete | POST | Delete existing DC application |
|  | /dcapps | GET | Obtain the DC APP profile related information according to the APPID |

#### 6.1.3.2	Resource: DC APP

##### 6.1.3.2.1	Description

##### 6.1.3.2.2	Resource Definition

Resource URI: {apiRoot}/mmtel-dcappmgmt/<apiVersion>/dcapps

This resource shall support the resource URI variables defined in table 6.1.3.2.2-1.

Table 6.1.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |

##### 6.1.3.2.3	Resource Standard Methods

6.1.3.2.3.1	GET

Table 6.1.3.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.3.2.3.1-3.

Table 6.1.3.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DcAppIdReq | M | 1 | Represents the requested DC application information. |

Table 6.1.3.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DcAppIdResp | M | 1 | 200 OK | The requested DC application profile related information is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MMTel Enabler Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MMTel Enabler Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.3.2.3.1-4: Headers supported by the 307 method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MMTel Enabler Server. |

Table 6.1.3.2.3.1-5: Headers supported by the 308 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MMTel Enabler Server. |

##### 6.1.3.2.4	Resource Custom Operations

6.1.3.2.4.1	Overview

Table 6.1.3.2.4.1-1: Custom operations

| Operation name | Custom operaration URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| Configure | /dcapps/configure | POST | Enables to configure DC application and DC application profile. |
| Update | /dcapps/update | POST | Enables to update existing DC application and DC application profile. |
| Delete | /dcapps/delete | POST | Enables to delete existing DC application and DC application profile. |

6.1.3.2.4.2	Operation: Configure

This custom operation enables to configure DC application and DC application profile.

This operation shall support the request data structures specified in table 6.1.3.2.4.2-1 and the response data structure and response codes specified in table 6.1.3.2.4.2-2.

Table 6.1.3.2.4.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DcAppConfigReq | M | 1 | Represents the DC application and DC application profile configuration information. |

Table 6.1.3.2.4.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DcAppConfigResp | M | 1 | 201 Created | Indicates the successfully configured list of APPID. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MMTel Enabler Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MMTel Enabler Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codse for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.3.2.4.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MMTel Enabler Server. |

Table 6.1.3.2.4.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MMTel Enabler Server. |

6.1.3.2.4.3	Operation: Update

This custom operation enables to update existing DC application and DC application profile.

This operation shall support the request data structures specified in table 6.1.3.2.4.3-1 and the response data structure and response codes specified in table 6.1.3.2.4.3-2.

Table 6.1.3.2.4.3-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DcAppUpdateReq | M | 1 | Represents the DC application and DC application profile information that shall be updated. |

Table 6.1.3.2.4.3-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DcAppStatResp | M | 1 | 200 OK | Indicates the successfully updated list of APPID. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MMTel Enabler Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MMTel Enabler Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] shall also apply. |  |  |  |  |

Table 6.1.3.2.4.3-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MMTel Enabler Server. |

Table 6.1.3.2.4.3-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MMTel Enabler Server. |

6.1.3.2.4.4	Operation: Delete

This custom operation enables to delete existing DC application.

This operation shall support the request data structures specified in table 6.1.3.2.4.4-1 and the response data structure and response codes specified in table 6.1.3.2.4.4-2.

Table 6.1.3.2.4.4-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DcAppIdReq | M | 1 | Represents the DC application and DC application that shall be deleted. |

Table 6.1.3.2.4.4-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DcAppIdResp | M | 1 | 200 OK | Indicates the successfully deleted list of APPID. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MMTel Enabler Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative MMTel Enabler Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.3.2.4.4-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MMTel Enabler Server. |

Table 6.1.3.2.4.4-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative MMTel Enabler Server. |

### 6.1.4	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

### 6.1.5	Notifications

There are no custom operations without associated resources defined for this API in this release of the specification.

### 6.1.6	Data Model

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined specifically for the MMTel_DCAppManagement API service.

Table 6.1.6.1-1: MMTel_DCAppManagement API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| DcAppConfigReq | 6.1.6.2.2 | Represents the DC application and profile configuration request |  |
| DcAppConfigParameters | 6.1.6.2.3 | Represents the parameters of single-DC applicaion in the configuration request |  |
| DcAppConfigResp | 6.1.6.2.4 | Represents the DC application and profile configuration response |  |
| DcAppConfigResponseParameters | 6.2.6.2.5 | Represents the parameters of single-DC applicaion in the configuration response |  |
| DcAppUpdateReq | 6.1.6.2.6 | Represents the DC application and profile update request |  |
| DcAppUpdateParameters | 6.1.6.2.7 | Represents the parameters of single-DC applicaion in the update response |  |
| DcAppStatResp | 6.1.6.2.8 | Represents the DC application and profile update response or DC application delete response |  |
| DcAppResponseParameters | 6.1.6.2.9 | Represents the parameters of single-DC applicaion in the DC application and profile update response and DC application delete response |  |
| DcAppIdReq | 6.1.6.2.10 | Represents the DC application delete request and DC application information retrieval request |  |
| DcAppIdResp | 6.1.6.2.11 | Represents the DC application profile information retrieval response |  |
| Condition | 6.1.6.2.12 | Represents the conditions used by the DC application |  |
| AppLoadPhase | 6.1.6.3.3 | Represents the load phase of the DC application |  |
| SupportScenario | 6.1.6.3.4 | Represents the support scenario of the DC application |  |
| Status | 6.1.6.3.5 | Represents the request return status |  |

Table 6.1.6.1-2 specifies data types re-used by theMMTel_DCAppManagement API service.

Table 6.1.6.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Uri | 3GPP TS 29.122 [3] | Used to indicate the notification URI. |  |
| DateTime | 3GPP TS 29.122 [3] | Used to represent the subscription duration. |  |

#### 6.1.6.2	Structured data types

##### 6.1.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 6.1.6.2.2	Type: DcAppConfigReq

Table 6.1.6.2.2-1: Definition of type DcAppConfigReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| reqId | string | M | 1 | The identity of the DC Application provider |  |
| secCred | string | O | 0..1 | Security information required by the MMTel Enabler Server. |  |
| dcAppNum | integer | M | 1 | Indicates the total number of DC Application. |  |
| dcAppConfigParamList | array(DcAppConfigParameters) | M | 1..N | The list of DC application configuration parameters. |  |

##### 6.1.6.2.3	Type: DcAppConfigParameters

Table 6.1.6.2.3-1: Definition of type DcAppConfigParameters

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| appIndex | string | M | 1 | The index of the DC Application configuration parameter. |  |
| appName | string | O | 0..1 | The name of the Data Channel Application. |  |
| svcType | string | O | 0..1 | The service type of the Data Channel Application. |  |
| appIconUrl | Uri | O | 0..1 | The url of the Data Channel Application icon. |  |
| appVer | string | O | 0..1 | The latest version of the DC application in UE. |  |
| appVal | DateTime | O | 0..1 | The latest validity of the DC application. |  |
| appLoadPh | AppLoadPhase | O | 0..1 | Indicates the phase when this Data Channel Application is allowed to be used. |  |
| autoload | boolean | O | 0..1 | Indicates whether this Data Channel Application is needed to be load to the UE automatically. Set to "true" if this Data Channel Application is needed to be load to the UE automatically. otherwise set to "false". Default value is "false". |  |
| autolaunch | boolean | O | 0..1 | Indicates whether this Data Channel Application is needed to be downloaded to the UE and run automatically. Set to "true" if this Data Channel Application is needed to be downloaded to the UE and run automatically. otherwise set to "false". Default value is "false". |  |
| peerDcReq | boolean | O | 0..1 | Indicates whether this Data Channel Application can be used if Data Channel is not supported by the other party of the call. Set to "true" if this Data Channel Application can be used if Data Channel is not supported by the other party of the call. otherwise set to "false". Default value is "false". |  |
| suppScnr | SupportScenario | O | 0..1 | Indicates supported media type required for this Data Channel Application. |  |
| cond | array(Condition) | O | 0..N | Indicates whether this Data Channel Application can be used if under the conditions |  |
| qosReq | string | O | 0..1 | Indicates the QoS requirement of this Data Channel Application. |  |
| persDataColl | boolean | O | 0..1 | Indicates whether this Data Channel Application will collect the user personal data. Set to "true" if this Data Channel Application will collect the user personal data. otherwise set to "false". Default value is "false". |  |
| persDataCollInfoUrl | Uri | O | 0..1 | The url of the purpose description of mandatory and optional personal data to be collected. |  |
| appPkg | Binary | O | 0..1 | The package of the Data Channel Application. |  |

##### 6.1.6.2.4	Type: DcAppConfigResp

Table 6.1.6.2.4-1: Definition of type DcAppConfigResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dcAppConfigRespList | array(DcAppConfigResponseParameters) | M | 1..N | The list of DC application configuration response parameters. |  |

##### 6.1.6.2.5	Type: DcAppConfigResponseParameters

Table 6.1.6.2.5-1: Definition of type DcAppConfigResponseParameters

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| appIndex | string | M | 1 | The index of the DC Application configuration parameter. |  |
| status | Status | M | 1 | Indicates the success or failure of the request. |  |
| appId | string | C | 0..1 | The unique identifier of the Data Channel Application assigned by the MMTel Enabler Server. May only be present if the status sets to "SUCCESS". |  |
| failureCause | string | C | 0..1 | The failure reason the DC Application configuration. May only be present if the status sets to "FAILED". |  |

##### 6.1.6.2.6	Type: DcAppUpdateReq

Table 6.1.6.2.6-1: Definition of type DcAppUpdateReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| reqId | string | M | 1 | The identity of the DC Application provider |  |
| secCred | string | O | 0..1 | Security information required by the MMTel Enabler Server. |  |
| dcAppNum | integer | M | 1 | Indicates the total number of DC Application. |  |
| dcAppUpdateParamList | array(DcAppUpdateParameters) | M | 1..N | The list of DC application update parameters. |  |

##### 6.1.6.2.7	Type: DcAppUpdateParameters

Table 6.1.6.2.7-1: Definition of type DcAppUpdateParameters

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| appId | string | M | 1 | The unique identifier of the Data Channel Application assigned by the MMTel Enabler Server. |  |
| appName | string | O | 0..1 | The name of the Data Channel Application. |  |
| svcType | string | O | 0..1 | The service type of the Data Channel Application. |  |
| appIconUrl | Uri | O | 0..1 | The url of the Data Channel Application icon. |  |
| appVer | string | O | 0..1 | The latest version of the DC application in UE. |  |
| appVal | DateTime | O | 0..1 | The latest validity of the DC application. |  |
| appLoadPh | AppLoadPhase | O | 0..1 | Indicates the phase when this Data Channel Application is allowed to be used. |  |
| autoload | boolean | O | 0..1 | Indicates whether this Data Channel Application is needed to be load to the UE automatically. Set to "true" if this Data Channel Application is needed to be load to the UE automatically. otherwise set to "false". Default value is "false". |  |
| autolaunch | boolean | O | 0..1 | Indicates whether this Data Channel Application is needed to be downloaded to the UE and run automatically. Set to "true" if this Data Channel Application is needed to be downloaded to the UE and run automatically. otherwise set to "false". Default value is "false". |  |
| peerDcReq | boolean | O | 0..1 | Indicates whether this Data Channel Application can be used if Data Channel is not supported by the other party of the call. Set to "true" if this Data Channel Application can be used if Data Channel is not supported by the other party of the call. otherwise set to "false". Default value is "false". |  |
| suppScnr | SupportScenario | O | 0..1 | Indicates supported media type required for this Data Channel Application. |  |
| cond | Condition | O | 0..1 | Indicates whether this Data Channel Application can be used if under this condition |  |
| qosReq | string | O | 0..1 | Indicates the QoS requirement of this Data Channel Application. |  |
| persDataColl | boolean | O | 0..1 | Indicates whether this Data Channel Application will collect the user personal data. Set to "true" if this Data Channel Application will collect the user personal data. otherwise set to "false". Default value is "false". |  |
| persDataCollInfoUrl | Uri | O | 0..1 | The url of the purpose description of mandatory and optional personal data to be collected. |  |
| appPkg | Binary | O | 0..1 | The package of the Data Channel Application. |  |

##### 6.1.6.2.8	Type: DcAppStatResp

Table 6.1.6.2.8-1: Definition of type DcAppStatResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dcAppStatRespList | array(DcAppResponseParameters) | M | 1..N | The list of DC application update response parameters. |  |

##### 6.1.6.2.9	Type: DcAppResponseParameters

Table 6.1.6.2.9-1: Definition of type DcAppResponseParameters

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| appId | string | M | 1 | The unique identifier of the Data Channel Application assigned by the MMTel Enabler Server. |  |
| status | Status | M | 1 | Indicates the success or failure of the request. |  |
| failureCause | string | C | 0..1 | The failure reason the DC Application configuration. May only be present if the status sets to "FAILED". |  |

##### 6.1.6.2.10	Type: DcAppIdReq

Table 6.1.6.2.10-1: Definition of type DcAppIdReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| reqId | string | M | 1 | The identity of the DC Application provider |  |
| secCred | string | O | 0..1 | Security information required by the MMTel Enabler Server. |  |
| dcAppNum | integer | M | 1 | Indicates the total number of DC Application. |  |
| appIdList | array(string) | M | 1..N | The list of appId. |  |

##### 6.1.6.2.11	Type: DcAppIdResp

Table 6.1.6.2.11-1: Definition of type DcAppIdResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| status | Status | M | 1 | Indicates the success or failure of the request. |  |
| secCred | string | O | 0..1 | Security information required by the MMTel Enabler Server. |  |
| dcAppInfoList | array(DcAppUpdateParameters) | C | 1..N | The list of DC application profile information. May only be present if the status sets to " SUCCESS". |  |
| failureCause | string | C | 0..1 | The failure reason the DC Application configuration. May only be present if the status sets to "FAILED". |  |
| NOTE: The "appPkg" attribute within the DcAppUpdateParameters data type provided within this attriute is not applicable and shall not be present. |  |  |  |  |  |

##### 6.1.6.2.12	Type: Condition

Table 6.1.6.2.12-1: Definition of type Condition

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| CONDTY | The Data Channel Application is allowed to be used in this condition, e.g. Service area |  |
| CONDVA | The value of the CONFTY. |  |

#### 6.1.6.3	Simple data types and enumerations

##### 6.1.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.1.6.3.2	Simple data types

None.

##### 6.1.6.3.3	Enumeration: AppLoadPhase

Table 6.1.6.3.3-1: Enumeration AppLoadPhase

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| PRECALL_ONLY | The Data Channel Application is allowed to be used before the MMTel call session is established. |  |
| INCALL | The Data Channel Application is allowed to be used after the MMTel call session is established. |  |
| PRECALL_AND_INCALL | The Data Channel Application is allowed to be used during the entire Precall and incall. |  |

##### 6.1.6.3.4	Enumeration: SupportScenario

Table 6.1.6.3.4-1: Enumeration SupportScenario

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| VOICE_CALL_ONLY | The Data Channel Application can be used if and only if the corresponding call is a voice call. |  |
| VIDEO_CALL_ONLY | The Data Channel Application can be used if and only if the corresponding call is a video call. |  |
| VOICE_AND_VIDEO_CALL | The Data Channel Application can be used in both voice call and video call. |  |

##### 6.1.6.3.5	Enumeration: 	Status

Table 6.1.6.3.5-1: Enumeration Status

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| SUCCESS | Indicates that the request is processed successfully. |  |
| FAILED | Indicates that the request fails to be processed. |  |

### 6.1.7	Error Handling

#### 6.1.7.1	General

For the MMTel_DCAppManagement API, HTTP error responses shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [2]. Protocol errors and application errors specified in clause 5.2.6 of 3GPP TS 29.122 [2] shall be supported for the HTTP status codes specified in table 5.2.6-1 of 3GPP TS 29.122 [2].

In addition, the requirements in the following clauses are applicable for the MMTel_DCAppManagement API.

#### 6.1.7.2	Protocol Errors

No specific protocol errors for the MMTel_DCAppManagement API are specified.

#### 6.1.7.3	Application Errors

The application errors defined for the MMTel_DCAppManagement API are listed in Table 6.1.7.3-1.

Table 6.1.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |

### 6.1.8	Feature negotiation

The optional features in table 6.1.8-1 are defined for the MMTel_DCAppManagement API. They shall be negotiated using the extensibility mechanism defined in clause 5.2.7 of 3GPP TS 29.122 [2].

Table 6.1.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 6.1.9	Security

The provisions of clause 6 of 3GPP TS 29.122 [2] shall apply for the MMTel_DCAppManagement API.

## 6.2	MMTel_DCAppCall API

And so on if there are more than two services supported by the MMTel Enabler Server. Same structure as in clause 6.1.

column.

## 6.3	MMTel_CallControlHandling API

And so on if there are more than two services supported by the MMTel Enabler Server. Same structure as in clause 6.1.

column.

## 6.4	<Service 4> API

And so on if there are more than two services supported by the MMTel Enabler Server. Same structure as in clause 6.1.

column.

# 7	Using Common API Framework

## 7.1	General

When CAPIF is used with a  MMTel Enabler Server service, the MMTel Enabler Server shall support the following functionalities as defined in 3GPP TS 29.222 [9]:

-	the API exposing function and the related APIs over CAPIF-2/2e and CAPIF-3/3e reference points;

-	the API publishing function and the related APIs over CAPIF-4/4e reference point;

-	the API management function and the related APIs over CAPIF-5/5e reference point; and

-	at least one of the security methods for authentication and authorization, and the related security mechanisms.

In a centralized deployment as defined in 3GPP TS 23.222 [8], where the CAPIF core function and the API provider domain functions are co-located, the interactions between the CAPIF core function and the API provider domain functions may be independent of the CAPIF-3/3e, CAPIF-4/4e and CAPIF-5/5e reference points.

When CAPIF is used with a MMTel Enabler Server service, the MMTel Enabler Server shall register all the northbound APIs features in the CAPIF Core Function.

## 7.2	Security

When CAPIF is used for external exposure, before invoking an API exposed by the MMTel Enabler Server, the service API consumer (e.g., Controlling Application Server) acting as an API invoker shall negotiate the security method (PKI, TLS-PSK or OAuth 2.0) with the CAPIF core function and ensure that the MMTel Enabler Server has enough credentials to authenticate the service API consumer (e.g. <provide examples of service consumers>), as defined in clauses 5.6.2.2 and 6.2.2.2 of 3GPP TS 29.222 [9].

If PKI or TLS-PSK is selected as the security method to be used between the service API consumer (e.g., Controlling Application Server) and the MMTel Enabler Server, upon API invocation, the MMTel Enabler Server  shall retrieve the authorization information from the CAPIF core function as described in clause 5.6.2.4 of 3GPP TS 29.222 [9].

As indicated in 3GPP TS 33.122 [10], the access to the MMTel Enabler Server APIs may be authorized by means of the OAuth 2.0 protocol (see IETF RFC 6749 [11]), using the "Client Credentials" authorization grant, where the CAPIF core function (see 3GPP TS 29.222 [9]) plays the role of the authorization server.

NOTE 1:	In this release, only "Client Credentials" authorization grant is supported.

If OAuth 2.0 is selected as the security method to be used between the service API consumer (e.g., Controlling Application Server) and the MMTel Enabler Server, the service API consumer (e.g., Controlling Application Server) shall, prior to consuming the services offered by the MMTel Enabler Server APIs, obtain a "token" from the authorization server, by invoking the Obtain_Authorization service operation as described in clause 5.6.2.3.2 of 3GPP TS 29.222 [9].

The MMTel Enabler Server APIs do not define any scopes for OAuth 2.0 authorization. It is the MMTel Enabler Server responsibility to check whether the service API consumer (e.g., Controlling Application Server) is authorized to use an API based on the provided "token". Once the MMTel Enabler Server verifies the "token", it shall check whether the MMTel Enabler Server identifier in the "token" matches its own published identifier, and whether the API name in the "token" matches its own published API name. If those checks are passed, the service API consumer (e.g., Controlling Application Server) has full authority to access any resource or operation provided by the invoked API.

NOTE 2:	For the aforementioned security methods, the MMTel Enabler Server needs to apply admission control according to access control policies after performing the authorization checks.

Annex A (normative):
OpenAPI specification

# A.1	General

This Annex specifies the formal definition of the API(s) defined in the present specification. It consists of OpenAPI specifications in YAML format.

This Annex takes precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API(s).

NOTE 1:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification files contained in this 3GPP Technical Specification are available on a Git-based repository that uses the GitLab software version control system (see clause 5.3.1 of 3GPP TS 29.501 [3] and clause 5B of 3GPP TR 21.900 [5]).

# A.2	MMTel_DCAppManagement API

Where <Service 1> is to be replaced by the name of the Service (e.g. MMTel_DCAppManagement).

One clause is introduced per Service, with the corresponding OpenAPI 3.0.0 Document.

openapi: 3.0.0

info:

title: <API Name>

version: 1.0.0-alpha.1

description: |

<API Name> Service.

© <2024>, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.<xxx> V<x.y.z>; <TS Name>.

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.xxx/

servers:

- url: '{apiRoot}/<API name in lower letters. Composed names are separated with a hyphen>/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122

security:

- {}

- oAuth2ClientCredentials: []

paths:

# API specific definitions, below is an example

/subscriptions:

post:

summary: subscribe to notifications

operationId: CreateIndividualSubcription

tags:

- Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/NsmfEventExposure'

responses:

'201':

description: Success

content:

application/json:

schema:

$ref: '#/components/schemas/<xxx>'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure:

{apiRoot}/<API name in lower letters. Composed names are separated with a hyphen>/<version>/subscriptions/{subId}

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

myNotification:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/<yyy>'

responses:

'204':

description: No Content, Notification was succesfull.

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subId}:

get:

summary: retrieve subscription

operationId: GetIndividualSubcription

tags:

- IndividualSubscription (Document)

parameters:

- name: subId

in: path

description: Event Subscription ID

required: true

schema:

type: string

responses:

'200':

description: OK. Resource representation is returned

content:

application/json:

schema:

$ref: '#/components/schemas/<xxx>'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: update subscription

operationId: ReplaceIndividualSubcription

tags:

- IndividualSubscription (Document)

tags:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/<xxx>'

parameters:

- name: subId

in: path

description: Event Subscription ID

required: true

schema:

type: string

responses:

'200':

description: OK. Resource was succesfully modified and representation is returned

content:

application/json:

schema:

$ref: '#/components/schemas/<xxx>'

'204':

description: No Content. Resource was succesfully modified

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: unsubscribe from notifications

operationId: DeleteIndividualSubcription

tags:

- IndividualSubscription (Document)

parameters:

- name: subId

in: path

description: Event Subscription ID

required: true

schema:

type: string

responses:

'204':

description: No Content. Resource was succesfully deleted

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

# API specific definitions

# A.3	MMTel_DCAppCall API

And so on if there are more than two services supported by the NF.

# A.4	MMTel_CallControlHandling API

And so on if there are more than two services supported by the NF.

# A.5	<Service 4> API

And so on if there are more than two services supported by the NF.

Annex <B> (informative):
Withdrawn API versions

# B.1	General

This Annex lists withdrawn API versions of the APIs defined in the present specification. 3GPP TS 29.501 [5] clause 4.3.1.6 describes the withdrawal of API versions.

# B.2	<Service 1> API

Where <Service 1> is to be replaced by the name of the Service (e.g. UAE_C2OperationModeManagement).

One clause is introduced per Service.

The API versions listed in table B.2-1 are withdrawn for the <Service 1> API.

Table B.2-1: Withdrawn API versions of the <Service 1> service

| API version number | Remarks |
| --- | --- |

In the Remarks column, the deficits leading to the withdrawal of an API version are explained.

# B.3	<Service 2> API

And so on if there are more than two services supported by the NF.

Annex <X> (informative):
Change history

This is the last annex for TS/TSs which details the change history using the following table.
This table is to be used for recording progress during the WG drafting process till TSG approval of this TS/TR.
For TRs under change control, use one line per approved Change Request
Date: use format YYYY-MM
CR: four digits, leading zeros as necessary
Rev: blank, or number (max two digits)
Cat: use one of the letters A, B, C, D, F
Subject/Comment: for TSs under change control, include full text of the subject field of the Change Request cover
New vers: use format [n]n.[n]n.[n]n

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-04 | CT3#140 | C3-251619 |  |  |  | Based on the skeleton for the new MMTel Enabler Server Services TS (C3-251557). Incorporates agreed pCRs C3-251558, C3-251559, C3-251560, C3-251561, C3-251562. Editorial changes and corrections from the rapporteur. | 0.1.0 |
| 2025-09 | CT3#142 | C3-253654 |  |  |  | Incorporates agreed pCRs C3-253338, C3-253455, C3-253456, C3-253457, C3-253458, C3-253604, C3-253704. Editorial changes and corrections from the rapporteur. | 0.2.0 |
| 2025-09 | CT#109 | CP-252069 |  |  |  | Presentation to TSG CT for information | 1.0.0 |