| 3GPP TS 29.673 V1.1.0 (2020-03) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; 5G System; UE Radio Capability Management Services Stage 3 (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	7

4	Overview	7

4.1	Introduction	7

5	Services offered by the UCMF	8

5.1	Introduction	8

5.2	Nucmf_UECapabilityManagement Service	8

5.2.1	Service Description	8

5.2.2	Service Operations	9

5.2.2.1	Introduction	9

5.2.2.2	Service Operation Resolve	9

5.2.2.2.1	General	9

5.2.2.2.2	Retrieve a Dictionary Entry	10

5.2.2.3	Service Operation Assign	10

5.2.2.4	Service Operation Subscribe	11

5.2.2.4.1	General	11

5.2.2.5	Unsubscribe	12

5.2.2.5.1	General	12

5.2.2.6	Notify	12

5.2.2.6.1	General	12

6	API Definitions	13

6.1	Nucmf_UECapabilityManagement Service API	13

6.1.1	Introduction	13

6.1.2	Usage of HTTP	13

6.1.2.1	General	13

6.1.2.2	HTTP standard headers	13

6.1.2.2.1	General	13

6.1.2.2.2	Content type	13

6.1.2.3	HTTP custom headers	14

6.1.2.4	HTTP multipart messages	14

6.1.3	Resources	15

6.1.3.1	Overview	15

6.1.3.2	Resource: Dictionary Entries	16

6.1.3.2.1	Description	16

6.1.3.2.2	Resource Definition	16

6.1.3.2.3	Resource Standard Methods	16

6.1.3.2.3.1	GET	16

6.1.3.2.3.2	POST	17

6.1.3.2.4	Resource Custom Operations	17

6.1.3.3	Resource: Individual Dictionary Entry	17

6.1.3.3.1	Description	17

6.1.3.3.2	Resource Definition	18

6.1.3.3.3	Resource Standard Methods	18

6.1.3.3.3.1	GET	18

6.1.3.4	Resource: Subscriptions collection	18

6.1.3.4.1	Description	18

6.1.3.4.2	Resource Definition	19

6.1.3.4.3	Resource Standard Methods	19

6.1.3.4.3.1	POST	19

6.1.3.4.4	Resource Custom Operations	19

6.1.3.5	Resource:  Individual subscription	19

6.1.3.5.1	Description	19

6.1.3.5.2	Resource Definition	19

6.1.3.5.3	Resource Standard Methods	20

6.1.3.5.3.2	DELETE	20

6.1.3.5.4	Resource Custom Operations	20

6.1.4	Custom Operations without associated resources	20

6.1.5	Notifications	20

6.1.5.1	General	20

6.1.5.2	UCMF Notification	21

6.1.5.2.1	Description	21

6.1.5.2.2	Target URI	21

6.1.5.2.3	Standard Methods	21

6.1.5.2.3.1	POST	21

6.1.6	Data Model	21

6.1.6.1	General	21

6.1.6.2	Structured data types	22

6.1.6.2.1	Introduction	22

6.1.6.2.2	Type: DicEntryData	22

6.1.6.2.3	Type: DicEntryCreateData	23

6.1.6.2.4	Type: DicEntryCreatedData	23

6.1.6.2.5	Type: UeRadioCapaId	23

6.1.6.2.6	Type: CreateSubscription	23

6.1.6.2.7	Type: CreatedSubscription	24

6.2.6.2.8	Type: UcmfNotification	24

6.2.6.2.9	Type: ManAssOpRequestlist	24

6.1.6.3	Simple data types and enumerations	24

6.1.6.3.1	Introduction	24

6.1.6.3.2	Simple data types	25

6.1.6.3.3	Enumeration: EventType	25

6.1.6.4	Data types describing alternative data types or combinations of data types	25

6.1.6.5	Binary data	25

6.1.6.5.2	UE Radio Capability Information	25

6.1.7	Error Handling	25

6.1.7.1	General	25

6.1.7.2	Protocol Errors	25

6.1.7.3	Application Errors	25

6.1.8	Feature negotiation	26

6.1.9	Security	26

Annex A (normative): OpenAPI specification	27

A.1	General	27

A.2	Nucmf_UECapabilityManagement API	27

Annex B (informative): Change history	34

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the stage 3 protocol and data model for the Nucmf Service Based Interface. It provides stage 3 protocol definitions and message flows, and specifies the API for each service offered by the Nucmf.

The 5G System stage 2 architecture and procedures are specified in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3].

The Technical Realization of the Service Based Architecture and the Principles and Guidelines for Services Definition are specified in 3GPP TS 29.500 [4] and 3GPP TS 29.501 [5].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	OpenAPI: "OpenAPI 3.0.0 Specification", https://github.com/OAI/OpenAPI-Specification/blob/master/versions/3.0.0.md.

[7]	3GPP TR 21.900: "Technical Specification Group working methods".

[8]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[9]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[10]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[11]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[12]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[13]	IETF RFC 7807: "Problem Details for HTTP APIs".

[14]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[15]	3GPP TS 38.413: "NG Radio Access Network (NG-RAN); NG Application Protocol (NGAP)".

[16]	3GPP TS 29.274: "3GPP Evolved Packet System (EPS); Evolved General Packet Radio Service (GPRS) Tunnelling Protocol for Control plane (GTPv2-C); Stage 3".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

void

## 3.2	Symbols

void

## 3.3	Abbreviations

void

# 4	Overview

## 4.1	Introduction

Within the 5GC, the UCMF offers services to the NF (e.g. AMF and MME) via the Nucmf service based interface (see 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3]).

Figure 4.1-1 provides the reference model (in service based interface representation and in reference point representation), with focus on the UCMF and the scope of the present specification.

Figure 4.1-1: Reference model – UCMF

The functionalities supported by the UCMF are listed in clause 6.2.5 of 3GPP TS 23.501 [2].

# 5	Services offered by the UCMF

## 5.1	Introduction

The UCMF supports the following services.

Table 5.1-1: NF Services provided by UCMF

| Service Name | Description | Example Consumer |
| --- | --- | --- |
| Nucmf_UECapabilityManagement | Allows the NF consumer to resolve UE Radio Capability ID (either Manufacturer-assigned or PLMN-assigned) into the corresponding UE radio access capability. Allows the NF consumer to obtain a PLMN-assigned UE Radio Capability ID for a specific UE radio access capability. Allows the NF consumer to subscribe or unsubscribe for notifications of UCMF dictionary entries. Allows the NF consumer to be notified about creation and deletion of UCMF dictionary entries. | AMF, MME |

## 5.2	Nucmf_UECapabilityManagement Service

### 5.2.1	Service Description

The Nucmf_UECapabilityManagement service operates on the dictionary entries for the mapping between UE Radio Capability ID and UE Radio Access Capability Information. The service operations exposed by this service allow service consumer NF, e.g. an AMF:

-	to retrieve UE radio access capability information with a UE Radio Capability ID (either Manufacturer-assigned or PLMN-assigned);

-	to obtain a PLMN-assigned UE Radio Capability ID for a specific UE radio access capability information;

-	to subscribe or unsubscribe for notifications of UCMF dictionary entries;

-	to be notified about creation and deletion of UCMF dictionary entries;

The Nucmf_UECapabilityManagement service supports the following service operations;

Table 5.2.1-1: Service operations supported by the Nucmf_UECapabilityManagement service

| Service Operations | Description | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Resolve | Retrieve UE radio access capability information from the dictionary entry identified by the UE Radio Capability ID (either Manufacturer-assigned or PLMN-assigned) | Request/Response | AMF, MME |
| Assign | Create a dictionary entry by supplying the UE Radio Access Capability Information for the current radio configuration of the UE and TAC, and obtain a PLMN-assigned UE Radio Capability ID. | Request/Response | AMF, MME |
| Subscribe | Subscribe for notifications of UCMF dictionary entries. | Subscribe/Notify | AMF, MME |
| Unsubscribe | Unsubscribe for notifications of UCMF dictionary entries. | Subscribe/Notify | AMF, MME |
| Notify | To be notified about creation and deletion of UCMF dictionary entries. | Subscribe/Notify | AMF, MME |

### 5.2.2	Service Operations

#### 5.2.2.1	Introduction

See Table 5.2.1-1 for an overview of the service operations supported by the Nucmf_UECapabilityManagement service.

#### 5.2.2.2	Service Operation Resolve

##### 5.2.2.2.1	General

The Resolve service operation shall be used to retrieve UE Radio Access Capability Information from a dictionary entry stored in the UCMF, by a NF service consumer, e.g. an AMF, when:

-	it has received an unknown UE Radio Capability ID, which is either Manufacturer-assigned or PLMN-assigned;

The NF Service Consumer (e.g. AMF) shall retrieve UE Radio Access Capability Information by using the HTTP GET method as shown in Figure 5.2.2.2.1-1.

Figure: 5.2.2.2.1-1 Retrieve UE Radio Access Capability Information

1.	The NF Service Consumer, e.g. an AMF, shall send an HTTP GET request to the resource URI Dictionary Entries collection resource. The query parameter is a Manufacturer-assigned or a PLMN-assigned UE Radio Capability ID.

2a.	On success, "200 OK" shall be returned. The response body shall contain UE Radio Access Capability Information stored in the dictionary entry that is matching the UE Radio Capability ID in query, and may contain a PLMN-assigned UE Radio Capability ID if the query parameter is a Manufacturer-assigned UE Radio Capability ID and if the PLMN-assigned UE Radio Capability ID mechanism is used in the network.

2b.	On failure, one of the HTTP status code listed in Table 6.1.3.3.3.2-3 shall be returned.  For a 4xx/5xx response, the message body may contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in Table 6.1.3.3.3.2-2, where applicable.

##### 5.2.2.2.2	Retrieve a Dictionary Entry

The Resolve service operation may be used to retrieve a dictionary entry stored in the UCMF, by a NF service consumer, e.g. an AMF, when it has the Dictionary Entry ID available.

The NF Service Consumer (e.g. AMF) shall retrieve a Dictionary Entry by using the HTTP GET method as shown in Figure 5.2.2.2.1-1.

Figure 5.2.2.2.1-1 Retrieve a dictionary entry

1.	The NF Service Consumer, e.g. an AMF, shall send an HTTP GET request to an individual Dictionary Entry resource.

2a.	On success, "200 OK" shall be returned. The response body shall the dictionary entry information for the given Dictionary Entry ID.

2b.	On failure, one of the HTTP status code listed in Table 6.1.3.3.3.2-3 shall be returned.  For a 4xx/5xx response, the message body may contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in Table 6.1.3.3.3.2-3, where applicable.

#### 5.2.2.3	Service Operation Assign

The Assign service operation shall be used to request the UCMF to create a new dictionary entry and assign a PLMN-assigned UE Radio Capability ID. The NF service consumer, e.g. an AMF, may consume the service by providing the UE Radio Access Capability Information retrieved from the UE, and Type Allocation Code of PEI of the UE, e.g. when it hasn't received any UE Radio Capability ID.

The NF Service Consumer (e.g. AMF) shall obtain the PLMN Assigned UE Radio Capability ID by using the HTTP POST method as shown in Figure 5.2.2.3.1-1.

Figure 5.2.2.3-1 Create a dictionary entry

1.	The NF Service Consumer shall send a POST request to the resource representing the Dictionary Entries collection resource of the UCMF. The payload body of the POST request shall contain:

-	a representation of the individual dictionary entry resource to be created;

-	the UE Radio Access Capability Information for the current radio configuration of the UE;

-	the Type Allocation Code of the PEI of the UE.

2a.	On success, "201 Created" shall be returned, the payload body of the POST response shall contain the representation describing the status of the request and the "Location" header shall be present and shall contain the URI of the created resource. When PLMN Assigned UE Radio Capability ID is used, the PLMN Assigned UE Radio Capability ID shall be included in DicEntryCreatedData.

2b.	On failure, one of the HTTP status code listed in Table 6.1.3.2.3.2-3 shall be returned.  For a 4xx/5xx response, the message body may contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in Table 6.1.3.3.3.2-2, where applicable.

#### 5.2.2.4	Service Operation Subscribe

##### 5.2.2.4.1	General

The Subscribe service operation shall be used by a NF Service Consumer, e.g. an AMF, to create a subscription in the UCMF, to get notifications for one or more new dictionary entries creation or for the deletion of one or more PLMN Assigned UE Radio Capability IDs.

The NF Service Consumer shall request to create a new subscription by using HTTP method POST with URI of the subscriptions collection, see clause 6.1.3.4. A ucmfNotificationUri shall be provided in the CreateSubscription to be used by the UCMF to send notifications later. See Figure 5.2.2.4.1-1.

Figure 5.2.2.4.1-1 Create a subscription

1.	The NF Service Consumer shall send a POST request to create a subscription resource in the UCMF. The payload body of the POST request shall contain a representation of the individual subscription resource to be created. The request may contain an expiry time, suggested by the NF Service Consumer as a hint, representing the time upto which the subscription is desired to be kept active and the time after which the subscribed event(s) shall stop generating report.

2a.	On success, the request is accepted, the UCMF shall include a HTTP Location header to provide the location of a newly created resource (subscription) together with the status code 201 indicating the requested resource is created in the response message. The UCMF may also include the highest dictionary entry ID which has been allocated.

The response, based on operator policy and taking into account the expiry time included in the request, may contain the expiry time, as determined by the UCMF, after which the subscription becomes invalid. Once the subscription expires, if the NF Service Consumer wants to keep receiving notifications, it shall create a new subscription in the UCMF. The UCMF shall not provide the same expiry time for many subscriptions in order to avoid all of them expiring and recreating the subscription at the same time. If the expiry time is not included in the response, the NF Service Consumer shall consider the subscription to be valid without an expiry time.

2b.	On failure or redirection, one of the HTTP status code listed in Table 6.1.3.4.3.1-3 shall be returned. For a 4xx/5xx response, the message body may contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in Table 6.1.3.4.3.1-3.

#### 5.2.2.5	Unsubscribe

##### 5.2.2.5.1	General

The Unsubscribe service operation is used by a NF Service Consumer, e.g. AMF, towards the UCMF, to remove an existing subscription previously created by itself at the UCMF.

The NF Service Consumer shall unsubscribe to the subscription by using HTTP method DELETE with the URI of the individual subscription resource (see clause 6.1.3.5) to be deleted. See Figure 5.2.2.5.1-1.

Figure 5.2.2.5.1-1 Unsubscribe a subscription

1.	The NF Service Consumer shall send a DELETE request to delete an existing subscription resource in the UCMF.

2a.	On success, the request is accepted, the UCMF shall reply with the status code 204 indicating the resource identified by subscription ID is successfully deleted in the response message.

2b.	On failure or redirection, one of the HTTP status code listed in Table 6.1.3.5.3.2-3 shall be returned. For a 4xx/5xx response, the message body may contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.3.5.3.2-3.

#### 5.2.2.6	Notify

##### 5.2.2.6.1	General

The Notify service operation is used by the UCMF, to send a notification, towards the notification URI, when certain event included in the subscription has taken place.

The UCMF shall use the HTTP method POST, using the notification URI received in the subscription creation as specified in clause 5.2.2.4.1. See Figure 5.2.2.6.1-1.

Figure 5.2.2.6.1-1 Notify

1.	The AMF shall send a POST request to send a notification.

2a.	On success, "204 No content" shall be returned by the NF Service Consumer.

2b.	On failure, the appropriate HTTP status code (e.g. "403 Forbidden") indicating the error shall be returned and appropriate additional error information should be returned.

# 6	API Definitions

## 6.1	Nucmf_UECapabilityManagement Service API

### 6.1.1	Introduction

The Nucmf_UECapabilityManagement service shall use the Nucmf_UECapabilityManagement API.

The request URI used in HTTP request from the NF service consumer towards the NF service producer shall have the structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/{apiVersion}/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [5].

-	The <apiName> shall be "nucmf-uecm".

-	The {apiVersion} shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 5.3.

### 6.1.2	Usage of HTTP

#### 6.1.2.1	General

HTTP/2, IETF RFC 7540 [11], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

The OpenAPI [6] specification of HTTP messages and content bodies for the <API Name> API is contained in Annex A.

#### 6.1.2.2	HTTP standard headers

##### 6.1.2.2.1	General

See clause 5.2.2 of 3GPP TS 29.500 [4] for the usage of HTTP standard headers.

##### 6.1.2.2.2	Content type

JSON, IETF RFC 8259 [12], shall be used as content type of the HTTP bodies specified in the present specification as specified in clause 5.4 of 3GPP TS 29.500 [4]. The use of the JSON format shall be signalled by the content type "application/json".

"Problem Details" JSON object shall be used to indicate additional details of the error in a HTTP response body and shall be signalled by the content type "application/problem+json", as defined in IETF RFC 7807 [13].

Multipart messages shall also be supported (see clause 6.1.2.4) using the content type "multipart/related", comprising:

-	one JSON body part with the "application/json" content type; and

-	one or more binary body parts with 3gpp vendor specific content subtypes.

The 3gpp vendor specific content subtypes defined in Table 6.1.2.2.2-1 shall be supported.

Table 6.1.2.2.2-1: 3GPP vendor specific content subtypes

| content subtype | Description |
| --- | --- |
| vnd.3gpp.ngap | Binary encoded payload, encoding NG Application Protocol (NGAP) IEs, as specified in clause 9.3 of 3GPP TS 38.413 [15] (ASN.1 encoded). |
| NOTE: Using 3GPP vendor content subtypes allows to describe the nature of the opaque payload (e.g. NGAP or 5GS NAS information) without having to rely on metadata in the JSON payload. |  |

See clause 6.1.2.4 for the binary payloads supported in the binary body part of multipart messages.

#### 6.1.2.3	HTTP custom headers

The mandatory HTTP custom header fields specified in clause 5.2.3.2 of 3GPP TS 29.500 [4] shall be applicable.

#### 6.1.2.4	HTTP multipart messages

HTTP multipart messages shall be supported to transfer opaque UE Radio Access Capability Information, in the following service operations (and HTTP messages):

-	Resolve Response (GET Response);

-	Assign Request (POST);

HTTP multipart message shall include one JSON body part and one binary body part UeRadioCapability which comprising the OCTET STRING of UE Radio Capability IE (i.e. excluding the IEI, Criticality and octet string length indicator, see also Annex B of 3GPP TS 29.274 [16]) as specified in clause 9.3.1.74 of 3GPP TS 38.413 [15]).

The JSON body part shall be the "root" body part of the multipart message. It shall be encoded as the first body part of the multipart message. The "Start" parameter does not need to be included.

The multipart message shall include a "type" parameter (see IETF RFC 2387 [10]) specifying the media type of the root body part, i.e. "application/json".

NOTE:	The "root" body part (or "root" object) is the first body part the application processes when receiving a multipart/related message, see IETF RFC 2387 [10]. The default root is the first body within the multipart/related message. The "Start" parameter indicates the root body part, e.g. when this is not the first body part in the message.

For each binary body part in a HTTP multipart message, the binary body part shall include a Content-ID header (see IETF RFC 2045 [12]), and the JSON body part shall include an attribute, defined with the RefToBinaryData type, that contains the value of the Content-ID header field of the referenced binary body part.

### 6.1.3	Resources

#### 6.1.3.1	Overview

Figure 6.1.3.1-1: Resource URI structure of the Nucmf_UECapabilityManagement API

Table 6.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Dictionary Entries collection | /dic-entries | POST | Nucmf_UECapabilityManagement_Assign |
|  |  | GET | Nucmf_UECapabilityManagement_Resolve |
| Individual Dictionary Entry | /dic-entries/{dicEntryId} | GET | Nucmf_UECapabilityManagement_Resolve |
| Subscriptions collection | /subscriptions | POST | Nucmf_UECapabilityManagement_Subscribe |
| Individual subscription | /subscriptions/{subscriptionId} | DELETE | Nucmf_UECapabilityManagement_Unsubscribe |

#### 6.1.3.2	Resource: Dictionary Entries

##### 6.1.3.2.1	Description

This resource represents the collection of the individual dictionary entries created in the UCMF, where each individual dictionary entry includes the mapping information between UE Radio Capability ID and UE Radio Access Capability Information, Type Allocation Code and Software version of a PEI.

This resource is modelled with the Collection resource archetype (see clause C.2 of 3GPP TS 29.501 [5]).

##### 6.1.3.2.2	Resource Definition

Resource URI: {apiRoot}/nucmf-ucm/<apiVersion>/dic-entries

This resource shall support the resource URI variables defined in table 6.1.3.2.2-1.

Table 6.1.3.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.1.1 |
| apiVersion | See clause 6.1.1 |

##### 6.1.3.2.3	Resource Standard Methods

###### 6.1.3.2.3.1	GET

This operation retrieves the UE Radio Access Capability Information from a dictionary entry stored in the UCMF, by querying with a Manufacturer-assigned or PLMN-assigned UE Radio Capability ID, or the TAC and the SVN as part of the IMEI/IMEISV of the UE.

This method shall support the URI query parameters specified in table 6.1.3.2.3.1-1.

Table 6.1.3.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ue-radio-capa-id | UeRadioCapaId | C | 0..1 | This IE shall be present when either PLMN assigned or Manufacturer assigned UE Radio Capability ID is used to retrieve a dictionary entry. When present, it shall contain a UE Radio Capability ID. (NOTE) |  |

This method shall support the request data structures specified in table 6.1.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.3.2.3.1-3.

Table 6.1.3.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DicEntryData | M | 1 | 200 OK | The response body contains a dictionary entry that is matching the querying parameter. |
| ProblemDetails | O | 1 | 400 Bad Request | The response body contains the error reason of the request message. |
| ProblemDetails | O | 1 | 404 Not Found | The "cause" attribute may be used to indicate the following application error: - NO_DICTIONARY_ENTRY_FOUND See table 6.1.7.3-1 for the description of this error. |
| NOTE: The manadatory HTTP error status code for the GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply, with response body containing an object of ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). |  |  |  |  |

###### 6.1.3.2.3.2	POST

This method creates an individual dictionary entry resource in the UCMF.

This method shall support the URI query parameters specified in table 6.1.3.2.3.2-1.

Table 6.1.3.2.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.2.3.2-2 and the response data structures and response codes specified in table 6.1.3.2.3.2-3.

Table 6.1.3.2.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DicEntryCreateData | M | 1 | Representation of the Dictionary Entry Data to be created in the UCMF. |

Table 6.1.3.2.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DicEntryCreatedData | M | 1 | 201 Created | Successfully a dictionary entry is created. |
| ProblemDetails | M | 1 | 400 Bad Request | The response body contains the error reason of the request message. |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

##### 6.1.3.2.4	Resource Custom Operations

None.

#### 6.1.3.3	Resource: Individual Dictionary Entry

##### 6.1.3.3.1	Description

This resource represents an individual Dictionary Entry for the mapping information between UE Radio Capability ID(s) and UE Radio Access Capability information, identified by the Dictionary Entry ID.

This resource is modelled as the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [5]).

##### 6.1.3.3.2	Resource Definition

Resource URI:{apiRoot}/nucmf-ucm/<apiVersion>/dic-entries/{dicEntryId}

This resource shall support the resource URI variables defined in table 6.1.3.3.2-1.

Table 6.1.3.3.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.1.1 |
| apiVersion | See clause 6.1.1. |
| dicEntryId | Integer with range 1-4294967295. |

##### 6.1.3.3.3	Resource Standard Methods

###### 6.1.3.3.3.1	GET

This operation retrieves an individual dictionary entry resource for the mapping information between UE Radio Capability ID(s) and UE Radio Access Capability information, stored in the UCMF, identified by the dicEntryId.

This method shall support the URI query parameters specified in table 6.1.3.2.3.1-1.

Table 6.1.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 6.1.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.3.2.3.1-3.

Table 6.1.3.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DicEntryData | M | 1 | 200 OK | The response body contains a dictionary entry for the given Dictionary Entry ID. |
| ProblemDetails | O | 1 | 404 Not Found | The "cause" attribute may be used to indicate the following application error: - NO_DICTIONARY_ENTRY_FOUND See table 6.1.7.3-1 for the description of this error. |
| NOTE: The manadatory HTTP error status code for the GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply, with response body containing an object of ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). |  |  |  |  |

#### 6.1.3.4	Resource: Subscriptions collection

##### 6.1.3.4.1	Description

This resource represents a collection of subscriptions in the UCMF, created by NF service consumers of Nucmf_UECapabilityManagement service.

This resource is modelled as the Collection resource archetype (see clause C.2 of 3GPP TS 29.501 [5]).

##### 6.1.3.4.2	Resource Definition

Resource URI: {apiRoot}/nucmf-ucm/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in table 6.1.3.4.2-1.

Table 6.1.3.4.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.1.1 |
| apiVersion | See clause 6.1.1. |

##### 6.1.3.4.3	Resource Standard Methods

###### 6.1.3.4.3.1	POST

This method shall support the URI query parameters specified in table 6.1.3.4.3.1-1.

Table 6.1.3.4.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.4.3.1-2 and the response data structures and response codes specified in table 6.1.3.4.3.1-3.

Table 6.1.3.4.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| CreateSubscription | M | 1 | The Subscription to be created by a NF Service consumer, e.g. an AMF. |

Table 6.1.3.4.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| CreatedSubscription | M | 1 | 201 Created | Represents successful creation of a Subscription in the UCMF |
| ProblemDetails | O | 1 | 400 Bad Request | The response body contains the error reason of the request message. |

##### 6.1.3.4.4	Resource Custom Operations

None.

#### 6.1.3.5	Resource:  Individual subscription

##### 6.1.3.5.1	Description

This resource represents an individual of subscription in the UCMF, created earlier by a NF Service Consumer of Nucmf_UECapabilityManagement service.

This resource is modelled as the Document resource archetype (see clause C.1 of 3GPP TS 29.501 [5]).

##### 6.1.3.5.2	Resource Definition

Resource URI: {apiRoot}/nucmf-ucm/<apiVersion>/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 6.1.3.5.2-1.

Table 6.1.3.5.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.1.1 |
| apiVersion | See clause 6.1.1. |
| subscriptionId | String identifies an individual subscription in the UCMF. |

##### 6.1.3.5.3	Resource Standard Methods

###### 6.1.3.5.3.2	DELETE

This method shall support the URI query parameters specified in table 6.1.3.5.3.2-1.

Table 6.1.3.5.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.5.3.2-2 and the response data structures and response codes specified in table 6.1.3.5.3.2-3.

Table 6.1.3.5.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.5.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content |  |
| ProblemDetails | O | 1 | 404 Not Found | Indicates the modification of subscription has failed due to application error. The "cause" attribute may be used to indicate the following application error: - SUBSCRIPTION_NOT_FOUND. |

##### 6.1.3.5.4	Resource Custom Operations

None.

### 6.1.4	Custom Operations without associated resources

None.

### 6.1.5	Notifications

#### 6.1.5.1	General

Notifications shall comply to clause 6.2 of 3GPP TS 29.500 [4] and clause 4.6.2.3 of 3GPP TS 29.501 [5].

#### 6.1.5.2	UCMF Notification

##### 6.1.5.2.1	Description

The UCMF Notification is used by the UCMF to report one or several observed Events to a NF service consumer that has subscribed to such Notifications via the Individual Notification Subscription Resource.

##### 6.1.5.2.2	Target URI

The Notification URI "{ucmfNotificationUri}" shall be used with the resource URI variables defined in table 6.1.5.2.2-1.

Table 6.1.5.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| ucmfNotificationUri | String formatted as URI with the UCMF Notification Uri |

##### 6.1.5.2.3	Standard Methods

###### 6.1.5.2.3.1	POST

This method shall support the request data structures specified in table 6.1.5.2.3.1-1 and the response data structures and response codes specified in table 6.1.5.2.3.1-1.

Table 6.1.5.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| UcmfNotification | M | 1 | Represents the notification to be delivered |

Table 6.1.5.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content |  |
| NOTE: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

### 6.1.6	Data Model

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nucmf_UECapabilityManagement service based interface protocol.

Table 6.1.6.1-1: Nucmf_UECapabilityManagement specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| DicEntryData | 6.1.6.2.2 |  |  |
| DicEntryCreateData | 6.1.6.2.3 |  |  |
| DicEntryCreatedData | 6.1.6.2.4 |  |  |
| UeRadioCapaId | 6.1.6.2.5 |  |  |
| CreateSubscription | 6.1.6.2.6 |  |  |
| CreatedSubscription | 6.1.6.2.7 |  |  |
| UcmfNotification | 6.1.6.2.8 |  |  |
| ManAssOpRequestlist | 6.1.6.2.9 |  |  |

Table 6.1.6.1-2 specifies data types re-used by the Nucmf_UECapabilityManagement service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the N<NF> service based interface.

Table 6.1.6.1-2: Nucmf_UECapabilityManagement re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| PlmnAssiUeRadioCapId | 3GPP TS 29.571 [14] |  |  |
| ManAssiUeRadioCapId | 3GPP TS 29.571 [14] |  |  |
| TypeAllocationCode | 3GPP TS 29.571 [14] |  |  |
| NfInstanceId | 3GPP TS 29.571 [14] |  |  |
| DateTime | 3GPP TS 29.571 [14] |  |  |

#### 6.1.6.2	Structured data types

##### 6.1.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 6.1.6.2.2	Type: DicEntryData

Table 6.1.6.2.2-1: Definition of type DicEntryData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| plmnAssiUeRadioCapId | PlmnAssiUeRadioCapId | C | 0..1 | This IE shall include a PLMN Assigned UE Radio Capability ID if allocated in the dictionary entry. |
| manAssiUeRadioCapId | ManAssiUeRadioCapId | C | 0..1 | This IE shall include a Manufacturer Assigned UE Radio Capability ID if available in the dictionary entry. |
| typeAllocationCode | TypeAllocationCode | C | 0..1 | This IE shall contain the Type Allocation Code in corresponding to the UE Radio Access Capability in the dictionary entry. |
| ueRadioCapability | RefToBinaryData | M | 1 | This IE shall be included to contain the UE Radio Capability Information encoded as OCTET STRING of UE Radio Capability IE as specified in clause 9.3.1.74 of 3GPP TS 38.413 [15]). |
| NOTE: The information in the dictionary entry which is included as the query parameter(s) in the request message shall not be present. |  |  |  |  |

##### 6.1.6.2.3	Type: DicEntryCreateData

Table 6.1.6.2.3-1: Definition of type DicEntryCreateData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| typeAllocationCode | TypeAllocationCode | M | 1 | This IE shall contain the Type Allocation Code in corresponding to the UE Radio Access Capability in the dictionary entry. |
| ueRadioCapability | RefToBinaryData | M | 1 | This IE shall be included to contain the UE Radio Capability Information encoded as OCTET STRING of UE Radio Capability IE specified in clause 9.3.1.74 of 3GPP TS 38.413 [15]). |

##### 6.1.6.2.4	Type: DicEntryCreatedData

Table 6.1.6.2.4-1: Definition of type DicEntryCreateData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| plmnAssiUeRadioCapId | PlmnAssiUeRadioCapId | M | 1 | This IE shall include a PLMN Assigned UE Radio Capability ID if allocated. |

##### 6.1.6.2.5	Type: UeRadioCapaId

Table 6.1.6.2.5-1: Definition of type UeRadioCapaId

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| plmnAssiUeRadioCapId | PlmnAssiUeRadioCapId | C | 0..1 | This IE shall include a PLMN Assigned UE Radio Capability ID. (NOTE) |
| manAssiUeRadioCapId | ManAssiUeRadioCapId | C | 0..1 | This IE shall include a Manufacturer Assigned UE Radio Capability ID. (NOTE) |
| NOTE: Only one of plmnAssiUeRadioCapId and manAssiUeRadioCapId shall be present during retrieving UE Radio Access Capability Information. (See clause 6.1.3.2.3.1) |  |  |  |  |

##### 6.1.6.2.6	Type: CreateSubscription

Table 6.1.6.2.6-1: Definition of type CreateSubscription

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| nfId | NfInstanceId | C | 1 | This IE shall contain the NF Instance ID of the service consumer, e.g. an AMF, if it is available. |
| ucmfNotificationUri | Uri | M | 1 | This IE shall contain the callback URI on which the subscribed events shall be notified. |
| suggestedExpires | DateTime | O | 0..1 | If present, indicates the point in time at which the subscription expires, which is suggested by the service consumer. |

##### 6.1.6.2.7	Type: CreatedSubscription

Table 6.1.6.2.7-1: Definition of type CreatedSubscription

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| dicEntryId | DicEntryId | M | 1 | This IE shall contain the highest DicEntryId has been allocated in the UCMF. |
| confirmedExpires | DateTime | O | 0..1 | If present, indicates the point in time at which the subscription expires, which is confirmed by the service producer. |

##### 6.2.6.2.8	Type: UcmfNotification

Table 6.2.6.2.8-1: Definition of type UcmfNotification

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| dicEntryId | DicEntryId | M | 1 | This IE shall contain the highest DicEntryId has been allocated in the UCMF. |
| eventType | EventType | M | 1 | This IE shall contain the different events type included in the notification. |
| manAssOpRequestlist | ManAssOpRequestlist | C | 0..1 | This IE shall be present to contain the Manufacturer Assigned operation requested list which includes a list of PLMN Assigned UE Radio Capability IDs or a list of TACs if the event type indicates the deletion of one or more PLMN Assigned UE Radio Capability IDs as specified in 3GPP TS 23.501 [2]. |

##### 6.2.6.2.9	Type: ManAssOpRequestlist

Table 6.2.6.2.9-1: Definition of type DelPlmnAssiUeRadioCapIdData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| plmnAssiUeRadioCapId | array(PlmnAssiUeRadioCapId) | C | 0..N | This IE shall contain one or more PLMN Assigned UE Radio Capability IDs to be deleted. (NOTE) The UCMF shall always provide a complete list of PLMN Assigned UE Radio Capability IDs to enable the AMF to overwrite the existing list. |
| typeAllocationCode | array (TypeAllocationCode) | C | 0..N | This IE shall contain one or more Type Allocation Code in corresponding to the PLMN Assigned UE Radio Capability IDs to be deleted. (NOTE) The UCMF shall always provide a complete list of Type Allocation Codes to enable the AMF to overwrite the existing list. |
| NOTE: Either plmnAssiUeRadioCapId or typeAllocationCode shall be present, not for both. |  |  |  |  |

#### 6.1.6.3	Simple data types and enumerations

##### 6.1.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.1.6.3.2	Simple data types

The simple data types defined in table 6.1.6.3.2-1 shall be supported.

Table 6.1.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| DicEntryId | Integer | integer with range 1-4294967295. |  |

##### 6.1.6.3.3	Enumeration: EventType

The enumeration EventType represents different type of events included in a notification from the UCMF. It shall comply with the provisions defined in table 6.1.5.3.3-1.

Table 6.1.6.3.3-1: Enumeration EventType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "CREATION_OF_DICTIONARY_ENTRY" | New dictionary entries are created in the UCMF. |  |
| "DELETION_OF_PLMN_ASSIGNED_IDS" | One or more PLMN-assigned UE Radio Capability IDs are deleted. |  |

#### 6.1.6.4	Data types describing alternative data types or combinations of data types

None.

#### 6.1.6.5	Binary data

6.1.6.5.1	Introduction

This clause defines the binary data that shall be supported in a binary body part in an HTTP multipart message (see clauses 6.1.2.2.2 and 6.1.2.4).

##### 6.1.6.5.2	UE Radio Capability Information

UeRadioCapability shall be encoded with the OCTET STRING of UE Radio Capability IE (i.e. excluding the IEI, Criticality and octet string length indicator, see also Annex B of 3GPP TS 29.274 [16]) as specified in clause 9.3.1.74 of 3GPP TS 38.413 [15]), using the vnd.3gpp.ngap content-type.

### 6.1.7	Error Handling

#### 6.1.7.1	General

For the Nucmf_UECapabilityManagement  API, HTTP error responses shall be supported as specified in clause 4.8 of 3GPP TS 29.501 [5]. Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [4] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [4].

In addition, the requirements in the following clauses are applicable for the Nucmf_UECapabilityManagement  API.

#### 6.1.7.2	Protocol Errors

No specific procedures for the Nucmf_UECapabilityManagement  service are specified.

#### 6.1.7.3	Application Errors

The application errors defined for the Nucmf_UECapabilityManagement  service are listed in Table 6.1.7.3-1.

Table 6.1.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| NO_DICTIONARY_ENTRY_FOUND | 404 | There is no dictionary entry matching the UE Radio Capability ID in query. |

## 6.1.8	Feature negotiation

The optional features in table 6.1.8-1 are defined for the Nucmf_UECapabilityManagement API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 6.1.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

## 6.1.9	Security

As indicated in 3GPP TS 33.501 [8] and 3GPP TS 29.500 [4], the access to the Nucmf_UECapabilityManagement API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [9]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [10]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming services offered by the Nucmf_UECapabilityManagement API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [10], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Nucmf_UECapabilityManagement service.

The Nucmf_UECapabilityManagement API defines a single scope "nucmf_uecapabilitymanagement" for the entire service, and it does not define any additional scopes at resource or operation level.

Annex A (normative):
OpenAPI specification

## A.1	General

This Annex specifies the formal definition of the API(s) defined in the present specification. It consists of OpenAPI 3.0.0 specifications in YAML format.

This Annex takes precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API(s).

NOTE 1:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification files contained in this 3GPP Technical Specification are available on the public 3GPP file server in the following locations (see clause 5B of the 3GPP TR 21.900 [7] for further information):

-	https://www.3gpp.org/ftp/Specs/archive/OpenAPI/<Release>/, and

-	https://www.3gpp.org/ftp/Specs/<Plenary>/<Release>/OpenAPI/.

NOTE 2:	To fetch the OpenAPI specification file after CT#83 plenary meeting for Release 15 in the above links <Plenary> must be replaced with the date the CT Plenary occurs, in the form of year-month (yyyy-mm), e.g. for CT#83 meeting <Plenary> must be replaced with value "2019-03" and <Release> must be replaced with value "Rel-15".

## A.2	Nucmf_UECapabilityManagement API

openapi: 3.0.0

info:

title: Nucmf_UECapabilityManagement

version: 1.0.0.alpha-2

description: |

Nucmf_UECapabilityManagement Service.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.673 V16.0.0; 5G System; UE Radio Capability Management Services

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.673/

servers:

- url: '{apiRoot}/nucmf-uecm/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- nucmf-uecm

paths:

/dic-entries:

get:

summary: retrieve a dictionary entry matching query parameters

operationId: RetrieveDictionaryEntry

tags:

- Dictionary Entry (Store)

parameters:

- name: ue-radio-capa-id

in: query

description: UE Radio Capability ID, either PLMN Assigned or Manufacturer Assigned

schema:

$ref: '#/components/schemas/UeRadioCapaId'

responses:

'200':

description: Expected response to a valid request

content:

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/DicEntryData'

binaryDataUeRadioCapability:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataUeRadioCapability:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'501':

$ref: 'TS29571_CommonData.yaml#/components/responses/501'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

post:

summary: Create a dictionary entry in the UCMF

tags:

- a dictionary entry (Document)

operationId: CreateDictionaryEntry

requestBody:

content:

multipart/related:  # message with binary body part(s)

schema:

type: object

properties: # Request parts

jsonData:

$ref: '#/components/schemas/DicEntryCreateData'

binaryDataUeRadioCapability:

type: string

format: binary

encoding:

jsonData:

contentType:  application/json

binaryDataUeRadioCapability:

contentType:  application/vnd.3gpp.ngap

headers:

Content-Id:

schema:

type: string

required: true

responses:

'201':

description: Dictionary Created

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/nucmf-uecm/<apiVersion>/dic-entries/{dicEntryId}'

required: true

schema:

type: string

content:

application/json:

schema:

$ref: '#/components/schemas/DicEntryCreatedData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'501':

$ref: 'TS29571_CommonData.yaml#/components/responses/501'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/dic-entries/{dicEntryId}:

get:

summary: Get an individual dictionary entry via dicEntryId

operationId: GetDicEntry

tags:

- dic Entry Id (Document)

parameters:

- name: dicEntryId

in: path

description: the ID of a dictionary entry in the UCMF

required: true

schema:

$ref: '#/components/schemas/DicEntryId'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/DicEntryData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'501':

$ref: 'TS29571_CommonData.yaml#/components/responses/501'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscriptions:

post:

summary: Nucmf_UECapabilityManagement Subscribe service Operation

operationId: CreateIndividualSubcription

tags:

- Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/CreateSubscription'

responses:

'201':

description: Success

content:

application/json:

schema:

$ref: '#/components/schemas/CreatedSubscription'

headers:

Location:

description: 'Contains the URI of the newly created resource, according to the structure: {apiRoot}/nucmf-uecm/<apiVersion>/subscriptions/{subscriptionId}'

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

myNotification:

'{$request.body#/ucmfNotificationUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/UcmfNotification'

responses:

'204':

description: No Content, Notification was succesfull

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

delete:

summary: unsubscribe from notifications

operationId: DeleteIndividualSubcription

tags:

- IndividualSubscription (Document)

parameters:

- name: subscriptionId

in: path

description: Event Subscription ID

required: true

schema:

type: string

responses:

'204':

description: No Content. Resource was succesfully deleted

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nucmf-uecm: Access to the Nucmf_UECapabilityManagement API

schemas:

DicEntryData:

type: object

required:

- ueRadioCapability

properties:

typeAllocationCode:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TypeAllocationCode'

plmnAssiUeRadioCapId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnAssiUeRadioCapId'

manAssiUeRadioCapId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ManAssiUeRadioCapId'

ueRadioCapability:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

DicEntryCreateData:

type: object

required:

- typeAllocationCode

- ueRadioCapability

properties:

typeAllocationCode:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TypeAllocationCode'

ueRadioCapability:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

DicEntryCreatedData:

type: object

required:

- plmnAssiUeRadioCapId

properties:

plmnAssiUeRadioCapId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnAssiUeRadioCapId'

UeRadioCapaId:

type: object

properties:

plmnAssiUeRadioCapId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnAssiUeRadioCapId'

manAssiUeRadioCapId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ManAssiUeRadioCapId'

CreateSubscription:

type: object

required:

- ucmfNotificationUri

properties:

nfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

ucmfNotificationUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

suggestedExpires:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

CreatedSubscription:

type: object

required:

- dicEntryId

properties:

dicEntryId:

$ref: '#/components/schemas/DicEntryId'

confirmedExpires:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

UcmfNotification:

type: object

required:

- dicEntryId

- eventType

properties:

dicEntryId:

$ref: '#/components/schemas/DicEntryId'

eventType:

$ref: '#/components/schemas/EventType'

manAssOpRequestlist:

$ref: '#/components/schemas/manAssOpRequestlist'

minItems: 1

manAssOpRequestlist:

type: object

oneOf:

- required: [plmnAssiUeRadioCapId]

- required: [typeAllocationCod]

properties:

plmnAssiUeRadioCapId:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnAssiUeRadioCapId'

minItems: 1

typeAllocationCode:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TypeAllocationCode'

#

# SIMPLE DATA TYPES

#

DicEntryId:

type: integer

minimum: 0

maximum: 4294967295

#

# ENUMERATIONS

#

EventType:

anyOf:

- type: string

enum:

- CREATION_OF_DICTIONARY_ENTRY

- DELETION_OF_PLMN_ASSIGNED_IDS

- type: string

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-09 | CT4#93 | C4-193849 |  |  |  | Initial draft | 0.1.0 |
| 2019-10 | CT4#94 | C4-194356 |  |  |  | Subscribe, Unsubscribe and Notify | 0.2.0 |
| 2019-10 | CT4#94 | C4-194357 |  |  |  | Individual dictionary entry resource | 0.2.0 |
| 2019-11 | CT4#95 | C4-195436 |  |  |  | Data type and openAPI related to Resolve, Assign | 0.3.0 |
| 2019-11 | CT4#95 | C4-195438 |  |  |  | Data type and openAPI related to (un)Subscribe/Notify | 0.3.0 |
| 2019-12 | CT#86 | CP-193069 |  |  |  | TS presented for information | 1.0.0 |
| 2020-03 | CT4#96e | C4-200622 |  |  |  | UcmfNotification for deletion | 1.1.0 |
| 2020-03 | CT4#96e | C4-200623 |  |  |  | Other alignment with stage 2 requirements | 1.1.0 |