| 3GPP TS 29.674 V0.1.0 (2019-09) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Interface between the UCMF and the MME; Stage 3 (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	7

4	Protocol Stack	7

4.1	Introduction	7

4.2	UDP Header and Port Numbers	7

4.2.1	General	7

4.2.2	Request Message	7

4.2.3	Response Message	8

4.3	IP Header and IP Addresses	8

4.3.1	General	8

4.3.2	Request Message	8

4.3.3	Response Message	8

5	General description	8

5.1	Introduction	8

6	Procedures	8

6.1	Introduction	8

6.2	Node Related Procedures	9

6.2.1	General	9

6.2.2	Heartbeat Procedure	9

6.2.2.1	General	9

6.2.2.2	Heartbeat Request	9

6.2.2.3	Heartbeat Response	9

6.3	Session Related Procedures	9

6.3.1	General	9

6.3.2	Create Dictionary Entry Procedure	9

6.3.2.1	General	9

6.3.2.2	Procedures in the MME	9

6.3.2.3	Procedures in the UCMF	10

6.3.3	Query Dictionary Entry Procedure	10

6.3.3.1	General	10

6.3.3.2	Procedures in the MME	10

6.3.3.3	Procedures in the UCMF	10

7	Messages and Message Formats	11

7.1	Introduction	11

7.2	Transmission Order and Bit Definitions	11

7.3	Message Format	11

7.4	Message Types	11

7.5	S17-AP Messages	11

7.6	Error Handling	11

8	Information elements	11

8.1	Information Elements Format	11

8.2	Information Elements	11

Annex A (informative): Change history	12

# Foreword

This clause is mandatory; do not alter the text in any way.

In drafting the TS/TR pay particular attention to the use of modal auxiliary verbs!

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, certain modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

NOTE 1:	The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

NOTE 2:	The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

NOTE 3:	The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

NOTE 4:	The constructions "can" and "cannot" shall not to be used as substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

NOTE 5:	The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the UE Radio Capability Management Protocol used on the interface between the MME and UCMF as specified in 3GPP TS 23.401 [2]. In the related stage 2 specifications the prefix S17 is used for these common procedures realised by URCMP.

NOTE: As an alternative of URCMP, the MME could use SBI interface to access UCMF Services.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[3]	IETF RFC 768: "User Datagram Protocol".

[4]	IETF RFC 791: "INTERNET PROTOCOL".

[5]	IETF RFC 8200: "Internet Protocol, Version 6 (IPv6) Specification".

# 3	Definitions of terms, symbols and abbreviations

This clause and its three clauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

URCMP	UE Radio Capability Management Protocol

# 4	Protocol Stack

## 4.1	Introduction

The protocol stack for the control plane over the S17 reference points shall be as depicted in Figure 4.1-1. Clauses 4.2 and 4.3 further specify the related UDP and IP requirements.

Figure 4.1-1: Control Plane stack over S17

## 4.2	UDP Header and Port Numbers

### 4.2.1	General

A User Datagram Protocol (UDP) compliant with IETF RFC 768 [3] shall be used.

### 4.2.2	Request Message

The UDP Destination Port number for a Request message shall be XX. It is the registered port number for URCMP.

The UDP Source Port for a Request message is a locally allocated port number at the sending entity.

NOTE:	The locally allocated source port number can be reused for multiple Request messages.

### 4.2.3	Response Message

The UDP Destination Port value of a Response message shall be the value of the UDP Source Port of the corresponding Request message.

The UDP Source Port of a Response message shall be the value from the UDP Destination Port of the corresponding message.

## 4.3	IP Header and IP Addresses

### 4.3.1	General

In this clause, "IP" refers either to IPv4 as defined by IETF RFC 791 [4] or IPv6 as defined by IETF RFC 8200 [5]. A URCMP entity shall support both IPv4 and IPv6.

### 4.3.2	Request Message

The IP Destination Address of a Request message shall be an IP address of the peer entity.

The IP Source Address of a Request message shall be an IP address of the sending entity.

### 4.3.3	Response Message

The IP Destination Address of a Response message shall be copied from the IP Source Address of the corresponding Request message.

The IP Source Address of a Response message shall be copied from the IP destination address of the corresponding Request message.

# 5	General description

## 5.1	Introduction

This document describes the procedures, messages and information elements over the S17 reference point between the MME and the UCMF to support UE Radio Capability Signalling optimization as specified in clause 5.11.3A of 3GPP TS 23.401 [2].

The UE Radio Capability Management Protocol (URCMP) is used over S17 reference point.

# 6	Procedures

## 6.1	Introduction

The following clauses specify the procedures supported over the S17 reference points.

## 6.2	Node Related Procedures

### 6.2.1	General

The following clauses specify the node related procedures over the S17 reference points.

### 6.2.2	Heartbeat Procedure

#### 6.2.2.1	General

Two messages are specified for URCMP heartbeat procedure: Heartbeat Request and Heartbeat Response. The use of these messages is further specified in clause xx of 3GPP TS 23.007 [3] for EPC.

#### 6.2.2.2	Heartbeat Request

The MME or UCMF may send a Heartbeat Request on a path to the peer node to find out if it is alive. Heartbeat Request messages may be sent for each peer which is known by the sender, e.g. based on the configuration.

An MME or a UCMF shall be prepared to receive a Heartbeat Request at any time and it shall reply with a Heartbeat Response message.

#### 6.2.2.3	Heartbeat Response

The message shall be sent as a response to a received Heartbeat Request message.

## 6.3	Session Related Procedures

### 6.3.1	General

The following clauses describe the session related procedures over the S17 reference points. A URCMP session refers to a signalling transaction addressing a dictionary entry containing the mapping information between the UE Radio Capability ID(s) and UE Radio Access Information.

### 6.3.2	Create Dictionary Entry Procedure

#### 6.3.2.1	General

The URCMP Create Dictionary Entry procedure shall be used to create a dictionary entry in the UCMF, and obtain a PLMN assigned UE Radio Capability ID.

#### 6.3.2.2	Procedures in the MME

When the UE Radio Capability ID, either PLMN-assigned or Manufacturer-assigned, is not received by the MME from a UE, or if the MME receives a rejection response from the UCMF with the cause code "NO_DICTIONARY_ENTRY_FOUND" when it uses a Manufacturer-assigned UE Radio Capability ID to retrieve the corresponding UE Radio Access Capability Information, the MME shall initiate the Create Dictionary Entry Procedure by sending a Create Dictionary Entry Request message towards the UCMF including:

-	the UE Radio Access Capability Information, which is received from the eNB as specified in 3GPP TS 36.413 [3].

-	Type Allocation Code in the IMEI of the UE

-	Software Version in the IMEI if available

When the MME receives a Create Dictionary Entry Response message with a successful cause, the MME function shall continue with the procedure which triggered the Create Dictionary Entry procedure as specified in 3GPP TS 23.401 [2].

#### 6.3.2.3	Procedures in the UCMF

When the UCMF receives a Create Dictionary Entry Request message, it shall reply with a Create Dictionary Entry Response message.

On success, the UCMF shall create a new dictionary entry with received information and include the following information in the Create Dictionary Entry Response message;

-	a dictionary entry ID;

-	a PLMN assigned UE Radio Capability ID, if PLMN is configured to use a PLMN specific UE Radio Capability ID.

On failure, the UCMF shall:

-	return an appropriate error cause value, e.g. MANDATORY_IE_MISSING, discard all the received information and not create any dictionary entry.

### 6.3.3	Query Dictionary Entry Procedure

#### 6.3.3.1	General

The URCMP Query Dictionary Entry procedure shall be used to retrieve UE Radio Access Capability Information from a dictionary entry in the UCMF.

#### 6.3.3.2	Procedures in the MME

When the MME receives a UE Radio Capability ID from a UE or the source MME (during an inter MME mobility procedure), which is either PLMN-assigned or Manufacturer-assigned, while there is no corresponding UE Radio Access Capability Information stored in the MME, the MME shall initiate the Query Dictionary Entry Procedure by sending a Query Dictionary Entry Request message towards the UCMF including:

-	a PLMN-assigned UE Radio Capability ID;

-	or a Manufacturer-assigned UE Radio Capability ID;

When the MME receives a Query Dictionary Entry Response message with a successful cause, the MME function shall continue with the procedure which triggered the Create Dictionary Entry procedure as specified in 3GPP TS 23.401 [2].

#### 6.3.3.3	Procedures in the UCMF

When the UCMF receives a Query Dictionary Entry Request message, it shall look up the dictionary entries and reply with a Query Dictionary Entry Response message.

On success, the UCMF shall include the following information in the Query Dictionary Entry Response message:

-	the UE Radio Access Capability Information in correspondence to the querying UE Radio Capability ID included in the request message;

-	a PLMN specific UE Radio Capability ID if the querying UE Radio Capability ID is a Manufacturer-assigned UE Radio Capability ID and if PLMN is configured to use a PLMN specific UE Radio Capability ID.

On failure, the UCMF shall:

-	return an appropriate error cause value, e.g. NO_DICTIONARY_ENTRY_FOUND.

# 7	Messages and Message Formats

## 7.1	Introduction

Editor's Note:	The following clauses specify the messages and message formats supported over the S17 reference points.

## 7.2	Transmission Order and Bit Definitions

Editor's Note: S17AP messages will be transmitted in network octet order starting with octet 1 with the most significant bit sent first. The most significant bit of an octet in a PFCP message is bit 8. If a field in a PFCP message spans over several octets, the most significant bit is bit 8 of the octet with the lowest number, unless specified otherwise.

## 7.3	Message Format

Editor's Note:	to specify S17-AP message format

## 7.4	Message Types

Editor's Note:	to specify S17-AP message type, e.g. node level

## 7.5	S17-AP Messages

Editor's Note:	to specify S17-AP message content

## 7.6	Error Handling

# 8	Information elements

## 8.1	Information Elements Format

Editor's Note: this clause will specify the information element format and type.

## 8.2	Information Elements

Editor's Note:	this clause will specify the information elements defined for S17.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2019-09 | CT4#93 | C4-193850 |  |  | Intial draft |  | 0.1.0 |