| 3GPP TR 29.829 V0.3.0 (2021-02) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Service-based support for SMS in 5GC; (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	8

3.3	Abbreviations	8

4	Overall Requirements	8

5	Key Issues	8

5.1	Key Issue #1: <KI#1> SBI-based SM MT message transfer	8

5.2	Key Issue #2: <KI#2> SBI-based SM MO message transfer	9

5.3	Key Issue #3: <KI#3> Mechanism to select the target PLMN based on GPSI when using SBI	10

5.3.1	Description	10

6	Solutions	10

6.1	Solution#1: Solution for SBI-based MT SMS	10

6.1.1	Introduction	10

6.1.2	Successful MT SMS message transfer	11

6.1.3	Unsuccessful MT SMS message transfer	14

6.1.4	Alert SC message transfer	14

6.1.5	Impacts on services, entities and interfaces	15

6.2	Solution#2: <S#2> Solution for SBI-based MO SMS	16

6.2.1	Introduction	16

6.2.2	Successful MO SMS message transfer	16

6.2.3	Unsuccessful MO SMS message transfer	17

6.2.4	Impacts on services, entities and interfaces	17

6.3	Solution #3: Mechanism to select the target PLMN based on GPSI using NRF	18

6.3.1	Introduction	18

6.3.2	Description	18

6.3.3	Procedures	20

6.3.3.1	Scenario 1: Selection of target PLMN based on MSISDN belonging to local PLMN	20

6.3.3.2	Scenario 2: Selection of target PLMN based on MSISDN belonging to a different PLMN with NP	21

6.3.3.3	Scenario 3: Selection of target PLMN based on MSISDN belonging to a different PLMN without NP	24

6.3.3.4	Scenario 4: Selection of target PLMN based on MSISDN belonging to a different PLMN via transit PLMN	26

6.3.4	Impacts on services, entities and interfaces	28

6.4	Solution#4: MT SM transfer	28

6.4.1	Introduction	28

6.4.2	Transfer of MT SM based on SBI	29

6.4.3	Routing MT SM based on SBI	30

6.4.4	Transfer of Report to MT SM based on SBI	31

6.4.5	Impacts on services, entities and interfaces	32

6.5	Solution#5: MO SM transfer	32

6.5.1	Introduction	32

6.5.2	SMS-IWMSC Discovery and Selection	33

6.5.3	Transfer of MO SM based on SBI	34

6.5.4	Transfer of Report to MO SM based on SBI	34

6.5.5	Impacts on services, entities and interfaces	35

6.6	Solution#6: MT SM transfer via SMS Router/IP-SM-GW	35

6.6.1	Introduction	35

6.6.2	Routing MT SM via SMS Router/IP-SM-GW based on SBI	36

6.6.3	Get routing information from SMS Router/IP-SM-GW	38

6.6.4	Transfer of MT SM from SMS Router/IP-SM-GW to SMSF	38

6.6.5	Transfer of Report from SMSF to SMS Router/IP-SM-GW	39

6.6.6	Impacts on services, entities and interfaces	40

6.7	Solution#7: Unsuccessful MT SM transfer via SMS Router/IP-SM-GW	40

6.7.1	Introduction	40

6.7.2	Failure for MT SMS transfer in SMS Router/IP-SM-GW	40

6.7.3	Impacts on services, entities and interfaces	40

6.8	Solution#8: Unsuccessful MT SM transfer	41

6.8.1	Introduction	41

6.8.2	Failure for MT SM transfer in SMSF	41

6.8.3	Failure for MT SM transfer in UDM	43

6.8.4	Impacts on services, entities and interfaces	43

6.9	Solution#9: Alert	44

6.9.1	Introduction	44

6.9.2	Alert MS memory capacity available	46

6.9.3	Alert MS reachable	47

6.9.4	Impacts on services, entities and interfaces	48

6.10	Solution#10: Alert the recovery of UE memory for SMS	48

6.10.1	Introduction	48

6.10.2	UE memory capacity state store	50

6.10.3	UE memory capacity state query	51

6.10.4	UE memory capacity available event	51

6.10.5	Alert MS reachable	51

6.10.6	Impacts on services, entities and interfaces	51

6.11	Solution#11: Unsuccessful MO SM transfer	52

6.11.1	Introduction	52

6.11.2	Failure for MO SM transfer in SMS-IWMSC	52

6.11.3	Impacts on services, entities and interfaces	53

7	Evaluations and Conclusions	53

7.1	Evaluation and Conclusions of Solutions for Key Issue#1	53

7.2	Evaluation and Conclusions of Solutions for Key Issue#2	54

Annex <X> (informative):	Change history	55

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

In the Rel-16 architecture for 5G SMSoNAS (i.e. SMS over NAS), SMSF and the UDM are not able to expose service-based interfaces to an IP-SM-GW, SMS Router or SMS Center, As a result, SMSoNAS transport to/from the SMSF/UDM (e.g. towards IP-SM-GW, SMS Router, SMS Center, etc.) enabling to send MO / MT SMS is still carried out via legacy MAP or Diameter protocols.

This generates the following main issues:

-	No SBI-based interface is defined for enabling the retrieval of routing information from the UDM for the transfer of short messages (used by SMS-Router, IP-SM-GW, etc.).

-	No SBI-based interface is defined for MO / MT SMS from or to IP-SM-GW/SMS Router or any other NF that may want to send/receive SMS via interactions with SMSF.

-	MAP and/or Diameter have to be supported by the SMSF and the UDM in order to fully support 5G SMSoNAS. It is hence not possible to deploy a pure SBI-based 5GC if SMS has to be supported.

-	In the roaming scenarios, MO and MT SMS cannot benefit from the new inter-PLMN 5GC security framework based on the SEPP and the use of the secured N32 interface, relying instead on legacy MAP and/or Diameter SMS interfaces with all the well-known security vulnerabilities.

This Technical Report aims to study the key issues and requirements for implementing SBA/SBI paradigm and associated enhancements for 5G SMS and potential solutions to resolve the identified issues.

# 1	Scope

The present document identifies the SMS transfer in 5G network based on service-based interfaces, which allow SMSF and UDM to only support the SBI based interfaces in 5GC and avoid resorting to legacy MAP and Diameter interfaces within the PLMN and across PLMNs including roaming scenarios, identifies the key issues that are related to support of Service-based interfaces in 5GC, analyses potential solutions to address the key issues.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[3]	3GPP TS 23.501: "System architecture for the 5G System (5GS); Stage 2".

[4]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[5]	3GPP TS 23.632: "User data interworking, coexistence and migration; Stage 2".

[6]	3GPP TS 29.540: "5G System; SMS Services; Stage 3".

[7]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[8]	3GPP TS 24.011: "Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Gateway MSC For Short Message Service (SMS-GMSC): function of an MSC capable of receiving a short message from an SC, interrogating an HLR for routing information and SMS info, and delivering the short message to the VMSC or the SGSN of the recipient MS.

Interworking MSC For Short Message Service (SMS-IWMSC): function of an MSC capable of receiving a short message from within the PLMN and submitting it to the recipient SC.

IP-Short-Message-Gateway (IP-SM-GW): function responsible for protocol interworking between the IP-based UE and the SC.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

MO SMS	Mobile Originated Short Message Service

MT SMS	Mobile Terminated Short Message Service

NAS	Non-Access-Stratum

SMS	Short Message Service

SM MO	Short Message Mobile Originated

SM MT	Short Message Mobile Terminated

SMSF	Short Message Service Function

SMSoNAS	SMS over NAS

UDM	Unified Data Management

# 4	Overall Requirements

In order to remove dependency of legacy MAP and Diameter interfaces from SMSF and UDM for SMS service, following requirements shall be considered during the study:

-	A full SBI based solution for SMS service in 5GC shall be specified;

-	Routing mechanism for SMS delivery shall be based on service-based interfaces between 5GC and legacy SMS nodes (e.g. SMS-Router, IP-SM-GW, etc.);

-	The candidate solutions shall be applicable for roaming and non-roaming scenarios;

-	SMS delivery for MSISDN-less terminals (e.g. CIoT terminals) shall be supported.

# 5	Key Issues

## 5.1	Key Issue #1: <KI#1> SBI-based SM MT message transfer

This Key Issue explains modifications and enhancements needed for SBI-based SM MT, compared to the SM MT in 3GPP TS 23.040 [2]. As showed in the figure 5.1-1, this key issue includes:

-	Fundamental procedures of SBI-based SM MT within SMS, including:

-	Procedures for SBI-based MT Message Transfer;

-	Other SM MT procedures in 3GPP TS 23.040 [2] which are infected by SBI-based SM MT.

-	Node functionality enhancements needed for SBI-based SM MT, including:

-	Enhancements of SMS-GMSC;

-	Enhancements of SMS Router;

-	Enhancements of IP-SM-GW;

-	Enhancements of SMSF;

-	Enhancements of UDM;

-	Enhancements of NRF;

-	Enhancements of other related node functionalities.

-	Interface enhancements for SBI-based SM MT, including:

-	Interface 2: Interface between SMS-GMSC and UDM;

-	Interface 3: Interface between SMS-GMSC and SMS Router; Interface between SMS Router and SMSF;

-	Interface 4: Interface between SMS-GMSC and IP-SM-GW;

-	Interface 5: Interface between IP-SM-GW and SMSF;

-	Interface 6: Interface between UDM and SMS Router/SMSF;

-	Interface 7: Interface between UDM and IP-SM-GW

Other related interfaces.

Routing rules for SM MT.

Figure 5.1-1: Message transfer architecture for SM MT

## 5.2	Key Issue #2: <KI#2> SBI-based SM MO message transfer

This Key Issue explains modifications and enhancements needed for SBI-based SM MO, compared to the SM MO in 3GPP TS 23.040 [2]. As showed in the figure5.2-1, this key issue includes:

-	Fundamental procedures of SBI-based SM MO within SMS, including:

-	Procedures for SBI-based MO Message Transfer;

-	Other SM MO procedures in 3GPP TS 23.040 [2] which are infected by SBI-based SM MO;

-	Node functionality enhancements needed for SBI-based SM MO, including:

-	Enhancements of SMS-IWMSC;

-	Enhancements of SMSF;

-	Enhancements of NRF;

-	Enhancements of other related node functionalities;

-	Interface enhancements needed for SBI-based SM MO, including:

-	Interface 3: Interface between SMS-IWMSC and SMSF;

-	Other related interfaces;

-	Routing rules for SM MO.

Figure 5.2-1: Message transfer architecture for SM MO

## 5.3	Key Issue #3: <KI#3> Mechanism to select the target PLMN based on GPSI when using SBI

### 5.3.1	Description

Short Message Service requires routing of some signalling messages based on the public identifier (e.g. MSISDN) of the SMS recipient.

This is the case of the interaction between the SMS-GMSC and the UDM to retrieve the routing information (i.e. the SMSF address) for the transfer of the Short Message to the SMSF serving the SMS recipient, where the signalling message is normally routed based on the recipient's MSISDN. In case that the SMS recipient belongs to a PLMN different from the PLMN of the SMS sender, the signaling takes place across PLMN borders.

Those interactions requiring routing based on MSISDN are currently accomplished using one of the existing protocols, MAP or Diameter, which can both use MSISDN as address.

When service-based interfaces are defined between the SMS entities (SMS-GMSC/IWMSC, SMS Router, IP-SM-GW) and the 5GC (UDM, SMSF) and the SMS entities and the 5GC are located in different PLMNs, it is needed a mechanism to determine the target PLMN for those SBI operations between the SMS entities and the 5GC requiring routing based on the GPSI of the SMS recipient, equivalent to existing routing methods based on MSISDN used in MAP or Diameter. Current service discovery mechanisms in SBA based on GPSI, specially discovery across PLMNs, do not include enough information to select the target PLMN. Additionally, Number Portability aspects also need to be considered.

Hence, it is needed to define a mechanism in 5GS to select the target PLMN for procedures involving service-based operations that require routing across PLMNs based on GPSI as public identifier.

This key issue aims at addressing the following aspects:

-	Study mechanisms to identify the PLMN where to route messages based on the recipient's GPSI for the applicable SBI operations between the SMS entities and the 5GC.

-	Consider Number Portability aspects to select the proper PLMN where the SMS recipient belongs to.

# 6	Solutions

## 6.1	Solution#1: Solution for SBI-based MT SMS

### 6.1.1	Introduction

This solution is to address Key Issue #1 "SBI-based SM MT message transfer". In order to remove dependency of legacy MAP and Diameter interfaces from SMSF and UDM for SMS service, corresponding services are defined, and node functionality enhancements are also introduced in this solution.

### 6.1.2	Successful MT SMS message transfer

A new service Nsmsf_SMSDelivery is defined, to be exposed by SMSF and registered in NRF, in order to e deliver MT SMS to SMSF, or to transfer the MO SMS Delivery Report to SMSF, as depicted in Figure 6.1.2-1.

Figure 6.1.2-1 MT SMS Delivery or MO SMS Delivery Report via the Nsmsf_SMSDelivery service

Another new service Nrouter_SMSSubmit is defined, to be exposed by SMS Router and registered in NRF, in order to enable the transfer of MT SMS Delivery Report to SMS Router.

Figure 6.1.2-2 MT SMS Delivery Report via the Nrouter_SMSSubmit service

Another new service Nudm_SmsRoutingInfo is defined, to be exposed by UDM and registered in NRF, in order to enable NF service consumer to get routing information from UDM, as depicted in Figure 6.1.2-3.

Figure 6.1.2-3 Routing information offered by the Nudm_SmsRoutingInfo service

Another new service Nrouter_SmsRoutingInfo is defined, to be exposed by SMS-Router and registered in NRF, in order to enable NF service consumer to get routing information from SMS Router, as depicted in Figure 6.1.2-4.

Figure 6.1.2-4 Routing information offered by the Nrouter_SmsRoutingInfo service

The procedure for successful SBI-based MT SMS message transfer is showed in Figure 6.1.2-5 and Figure 6.1.2-6, which is based on MT SMS procedures in 3GPP TS 23.040 [2] clause 10. Compared to procedures in 3GPP TS 23.040 [2], service providers register services in NRF, including:

-	Nudm_SmsRoutingInfo service provided by UDM.

-	Nrouter_SmsRoutingInfo service provided by SMS Router.

-	Nrouter_SMSSubmit service provided by SMS Router.

-	Nsmsf_SMSDelivery service provided by SMSF.

At the same time, service consumers discover services from NRF, including:

-	SMS-GMSC discovers Nudm_SmsRoutingInfo service from NRF.

-	SMS Router discovers Nudm_SmsRoutingInfo service from NRF.

-	UDM discovers Nrouter_SmsRoutingInfo service from NRF.

-	SMS-GMSC discovers Nrouter_SmsRoutingInfo service from NRF.

-	SMS Router discovers Nsmsf_SMSDelivery service from NRF.

-	SMS-GMSC discovers Nsmsf_SMSDelivery service from NRF.

-	SMSF discovers Nrouter_SMSSubmit service from NRF.

-	SMSF discovers Nsc_SMSSubmit service from NRF.

Figure 6.1.2-5: Procedures for successful SBI-based MT SMS message transfer (with SMS Router)

0	SMSF registers Nsmsf_SMSDelivery service in NRF, UDM registers Nudm_SmsRoutingInfo service in NRF, SMS Router registers Nrouter_SmsRoutingInfo and Nrouter_SmsRoutingInfo service service in NRF.

1	Message Transfer from SC to SMS-GMSC.

2a-2f	Alternative 1: the SMS-GMSC discovers and selects Nudm_SmsRoutingInfo to retrieve the routing information of the UE to be able to send the MT SM from UDM. The UDM discovers and selects Nrouter_SmsRoutingInfo to retrieve the address of the SMS Router to receive the MT SM, the UDM also provides the SMSF instance to the SMS Router. Then the SMS Router provides its address to the UDM to receive the MT SM. UDM provides the received address to the SMS-GMSC.

2g-2o	Alternative 2: the SMS-GMSC discovers and selects Nudm_SmsRoutingInfo to retrieve the routing information of the UE to be able to send the MT SM from UDM. The UDM responses the message with 3xx redirection, the URI of the SMS Router is included location header. After that, SMS-GMSC discovers and selects Nrouter_SmsRoutingInfo to retrieve the address of the SMS Router to receive the MT SM, and the SMS Router queries the UDM by invoking the Nudm_SmsRoutingInfo to retrieve the SMSF instance. The SMS Router/IP-SM-GW provides address to the SMS-GMSC to receive the MT SM.

3	SMS-GMSC forwards short message to SMSF through SMS Router.

4	SMS Router discovers Nsmsf_SMSDelivery service from NRF.

5	SMS Router to SMSF: Nsmsf_SMSDelivery service request, and MT SMS message transfer.

6	SMSF to SMS Router: Nsmsf_SMSDelivery service resoponse.

7	SMSF transfers message to UE.

8	SMSF to SMS Router: Nrouter_SMSSubmit service request, and transfer MT SMS Delivery Report.

9	SMS Router to SMSF: Nrouter_SMSSubmit service response.

10	SMS Router sends Delivery Report to SMS-GMSC.

11	SMS-GMSC sends SM-Delivery Report Status to UDM.

12	SMS-GMSC sends Delivery Report to SC.

Figure 6.1.2-6: Procedures for successful SBI-based MT SMS message transfer (without SMS Router)

0	SMSF registers Nsmsf_SMSDelivery service in NRF, UDM registers Nudm_SmsRoutingInfo service in NRF, SMS-GMSC registers Nsc_SMSSubmit service in NRF.

1	Message Transfer from SC to SMS-GMSC.

2	SMS-GMSC discovers and chooses Nudm_SmsRoutingInfo service from NRF.

3	SMS-GMSC to UDM: Nudm_SmsRoutingInfo service request.

4	UDM to GMSC: Nudm_SmsRoutingInfo service response.

5	SMS-GMSC discovers and chooses Nsmsf_SMSDelivery service from NRF.

6	SMS-GMSC to SMSF: Nsmsf_SMSDelivery service request, and forwards short message to SMSF directly.

7	SMSF to SMS-GMSC: Nsmsf_SMSDelivery service response.

8	SMSF transfers message to UE.

9	SMSF discovers and chooses Nsc_SMSSubmit service from NRF

10	SMSF to SMS-GMSC: Nsc_SMSSubmit service request, and transfer MT SMS Delivery Report.

11	SMS-GMSC to SMSF: Nsc_SMSSubmit service response.

12	SMS-GMSC sends SM-Delivery Report Status to UDM.

13	SMS-GMSC sends Delivery Report to SC.

Note:	Nsc_SMSSubmit service for MT SMS Delivery Report is defined in Solution#2 TR29.829.

SMS-GMSC selects UDM based on the GPSI of the UE, and UDM can choose the right SMS route information based on the GPSI.

Editor's note:	The Inter PLMN roaming case is FFS.

### 6.1.3	Unsuccessful MT SMS message transfer

Figure 6.1-1 Example of MT SMS delivery via the Nsmsf_SMSDelivery service

2b. If MT SMS delivery fails at the SMSF (e.g. the SUPI is unknown to the SMSF in the request body), the SMSF shall return an HTTP "400 Bad Request" status code with the ProblemDetails IE providing details of the error.

If MT SMS delivery fails at the SMSF due to SMSF internal errors, the SMSF shall return an HTTP "500 Internal Server Error" status code with the ProblemDetails IE providing details of the error.

The procedures defined in clause 4.13.3.9 of 3GPP TS 23.502 [4] and in TS 23.040 [2] shall then apply.

### 6.1.4	Alert SC message transfer

This solution proposes the definition of a new service Nsc_AlertSC, to be exposed by the SMS-IWMSC, in order to be alerted by the UDM when the UE is reachable for SMS (e.g. UE has responded to a paging request), when a delivery attempt has failed because the MS is not reachable or the MS memory capacity was exceeded. Consequently, The SC then may - on reception of an Alert-SC - initiate a new delivery attempt procedure for the queued messages destined for this MS.

Figure 6.1.4-1 Alert SC via Nsmsc_AlertSC service

### 6.1.5	Impacts on services, entities and interfaces

SMS-GMSC:

-	Support to discover Nudm_SmsRoutingInfo service from NRF.

-	Support to discover Nrouter_SmsRoutingInfo service from NRF.

-	Support to discover Nsmsf_SMSDelivery service from NRF.

-	Support to register Nsc_SmsRoutingInfo service from NRF.

-	Support SBI interface to UDM and SMSF.

UDM:

-	Support to discover Nrouter_SmsRoutingInfo service from NRF.

-	Support to register Nudm_SmsRoutingInfo service in NRF.

-	Support SBI interfaces to SMS-GMSC and SMS Router.

SMS Router:

-	Support to discover Nsmsf_SMSDelivery service from NRF.

-	Support to discover Nudm_SmsRoutingInfo service in NRF.

-	Support to register Nrouter_SmsRoutingInfo service in NRF.

-	Support to register Nrouter_SMSSubmit service in NRF.

-	Support SBI interfaces to UDM and SMSF.

SMSF:

-	Support to discover Nrouter_SMSSubmit service from NRF.

-	Support to discover Nsc_SMSSubmit service from NRF.

-	Support to register Nsmsf_SMSDelivery service in NRF.

-	Support SBI interfaces to SMS-GMSC and SMS Router.

NRF:

-	Support registration and discovery of the new defined services in this solution.

## 6.2	Solution#2: <S#2> Solution for SBI-based MO SMS

### 6.2.1	Introduction

This solution is to address Key Issue #2 "SBI-based SM MO message transfer". In order to remove dependency of legacy MAP and Diameter interfaces from SMSF and UDM for SMS service, corresponding services are defined, and node functionality enhancements are also introduced in this solution.

### 6.2.2	Successful MO SMS message transfer

A new service Nsc_SMSSubmit is defined, to be exposed by SMS-IWMSC/SMS-GMSC  and registered in NRF, in order to submit the MO SMS to SMS-IWMSC/SMS-GMSC, or to transfer the MT SMS Delivery Report to SMS-GMSC.

Figure 6.2.2-1 MO SMS Submit or MT SMS Delivery Report via the Nsc_SMSSubmit service

The procedure for successful SBI-based MO SMS message submit and MT SMS Delivery Report is showed in Figure 6.2.2-1, which is based on SMS procedures in 3GPP TS 23.040 [2] clause 10.2. Compared to procedures in 3GPP TS 23.040 [2], service providers register services in NRF, including:

-	Nsc_SMSSubmit service provided by SMS-IWMSC/SMS-GMSC.

-	Nsmsf_SMSDelivery service provided by SMSF.

At the same time, service consumers discover services from NRF, including:

-	SMSF discovers Nsc_SMSSubmit service from NRF.

-	SMS-IWMSC discovers Nsmsf_SMSDelivery service from NRF.

Figure 6.2.2-2: Procedures for successful SBI-based SM MO message transfer

0	SMS-IWMSC registers Nsc_SMSSubmit service in NRF, SMSF registers Nsmsf_SMSDelivery service in NRF.

1	Message Transfer from UE to SMSF through AMF.

2	SMSF discovers and chooses Nsc_SMSSubmit service for MO SMS submit from NRF.

3	SMSF to SMS-IWMSC: Nsc_SMSSubmit service request, and forwards MO SMS to SMS-IWMSC.

4	SMS-IWMSC to SMSF: Nsc_SMSSubmit service response.

5	SMS-IWMSC transfers message to SC.

6	Delivery Report from SC to SMS-IWMSC.

7	SMS-IWMSC discovers and chooses Nsmsf_SMSDelivery service for MO SMS Delivery Report from NRF.

8	SMS-IWMSC to SMSF: Nsmsf_SMSDelivery service request, and transfer MO SMS Delivery Report.

9	SMSF to SMS-IWMSC: Nsmsf_SMSDelivery service response.

10	Delivery Report from SMSF to UE.

Note:	Nsmsf_SMSDelivery service for MO SMS Delivery Report is defined in Solution#1 TR29.829.

SMSF can choose the right SMS-IWMSC base on SUPI/SUCI of the UE, if SMS-IWMSC locates in a different PLMN, SMSF can find the right SMS-IWMSC through vNRF and hNRF,

### 6.2.3	Unsuccessful MO SMS message transfer

Figure 6.2.2-1 MO SMS Submit via the Nsc_SMSSubmit service

2b. If MO SMS submit fails at the SC/SMS-IWMSC (e.g. the GPSI is unknown to the SC/SMS-IWMSC in the request body), the SC/SMS-IWMSC shall return an HTTP "403 Forbidden" status code with the ProblemDetails IE providing details of the error.

If MO SMS submit fails at the SC/SMS-IWMSC due to SC/SMS-IWMSC internal errors, the SC/SMS-IWMSC shall return an HTTP "500 Internal Server Error" status code with the ProblemDetails IE providing details of the error.

The procedures defined in TS 23.040 [2] shall then apply.

### 6.2.4	Impacts on services, entities and interfaces

SMS-IWMSC:

-	Support to register Nsc_SMSSubmit service in NRF.

-	Support to discover Nsmsf_SMSDelivery service from NRF.

-	Support SBI interface to SMSF.

-	Support to act as protocol converter between SBI and MAP/Diameter towards SC.

SMSF:

-	Support to register Nsmsf_SMSDelivery service from NRF.

-	Support to discover Nsc_SMSSubmit service from NRF.

-	Support SBI interfaces to SMS-IWMSC.

NRF:

-	Support registration and discovery of the newly defined services in this solution.

## 6.3	Solution #3: Mechanism to select the target PLMN based on GPSI using NRF

### 6.3.1	Introduction

This solution addresses Key Issue #3 ("Mechanism to select the target PLMN based on GPSI when using SBI").

Short Message Service requires routing of some signalling messages based on the public identifier (e.g. MSISDN) of the SMS recipient. In case that the SMS recipient belongs to a PLMN different from the PLMN of the SMS sender, the signaling takes place across PLMN borders.

Current service discovery mechanisms in SBA based on GPSI do not include information to select the target PLMN. This solution provides a mechanism when using service-based interfaces to select the target PLMN based on GPSI by enhancing the discovery and selection procedures using NRF and leveraging on the support of the existing DNS/ENUM and Number Portability services, if applicable, determining the target PLMN and whether SBI or legacy interfaces (e.g. MAP/Diameter) can be used. The proposal considers the selection of the target PLMN when the GPSI is an MSDIDN.

NOTE:	It is assumed that an External Identifier belongs to the home PLMN and hence it is not required to determine the target PLMN when the GPSI is an External Identifier.

While the solution is addressing the need for a new mechanism to select the target PLMN for those interactions between the SMS entities and the 5GC requiring routing based on the GPSI of the SMS recipient, it could be applicable to any potential use case and procedure requiring selection of the target PLMN based on GPSI in 5GS.

### 6.3.2	Description

The description of the solution is illustrated using the interaction between SMS-GMSC and UDM for MT SMS delivery in 5GC, i.e. a mechanism for the SMS-GMSC, acting as NF service consumer, to retrieve routing information from UDM in the target PLMN, acting as NF service producer, when service-based interfaces are used between the SMS entities and 5GC.

The solution proposes to select the target PLMN based on MSISDN using the discovery procedure with NRF according to the following:

-	The NF consumer (e.g. SMS-GMSC) located in the PLMN of subscriber A (SMS sender) performs discovery of the target NF/NF service instance(s) (e.g. UDM) invoking the local NRF in the same PLMN (referred as local PLMN hereinafter). The discovery request is based on the MSISDN of subscriber B (SMS recipient), which is the only known recipient's identity, and includes an optional indication for the NRF to determine the target PLMN and interface to be used.

-	The local NRF can determine the target PLMN and the interface to be used (SBI or legacy interface) performing an ENUM query, unless such information is available by other means, e.g. locally configured in the NRF.

-	An ENUM record for the full MSISDN or number series provisioned in the DNS/ENUM server can be used to indicate whether the user belongs to the local PLMN or another PLMN in the same or different country. The output of the lookup process using DNS/ENUM can be a URI that is provided in the ENUM response and points to the NRF in the target PLMN with which there is an interconnection agreement using SBI, so that the local NRF can send an inter-PLMN service discovery request to the target PLMN. An existing Enumservice type/subtype (e.g. "web:http") registered in IANA may be reused or new one(s) may be specified to be used for 5GC/SBA.

-	Absence of an ENUM record can imply to use a non-SBI interaction and hence a legacy interface should be used for the next operation request in the procedure (e.g. a MAP operation from SMS-GMSC towards HLR to retrieve routing information for SMS). This may be the case when the HSS/HLR is deployed in the target PLMN interworking with the UDM via UDICOM interface for SMS procedures, or there is no SBI interconnection with the target PLMN. An ENUM record containing an existing Enumservice type/subtype (e.g. pstn:tel) may be used as well to indicate that a non-SBI interface should be used.

-	If Number Portability (NP) service is required, the DNS/ENUM server can perform an NP query for national MSISDN number series according to the existing procedures. The answer from the NP service includes information about the target PLMN, e.g. the Routing Number (RN) assigned to the network operator owning that specific MSISDN, based on IETF RFC 4694 and local regulation in each country. The information provided by the NP service is included in the ENUM response for the local NRF to determine the target PLMN.

-	If NP is not required, the NRF can determine the target PLMN and the interface to be used (SBI or legacy interface) based on local configuration, instead of performing an ENUM query. If the NRF keeps local configuration of the MSISDN ranges or list of individual MSISDNs owned by the local PLMN, this information would need to be updated every time an MSISDN is added or removed to the list/ranges belonging to the PLMN. Additionally, the configuration may include information to determine the target PLMN for MSISDNs not belonging to the local PLMN.

-	The ENUM response may then include one of the following:

-	A URI pointing to the NRF of the target PLMN.

-	If NP is applicable, a RN associated to the network operator the MSISDN belongs to.

-	No record found for the MSISDN (or alternatively an ENUM record containing a specific Enumservice type/subtype), implying that SBI interactions should not be used.

-	Based on ENUM response or local configuration, the local NRF performs the following actions:

-	If the response contains information from NP such as RN, the local NRF uses the received information to determine the target PLMN (e.g. mapping the RN to PLMN ID).

-	If the user belongs to the local PLMN, the local NRF searches for NF/NF service instances (e.g. UDM instances) matching the discovery criteria that can serve the request using SBI interactions.

-	If the user belongs to a different PLMN, based on the URI received in the ENUM response pointing to the NRF in the target PLMN or based on the PLMN ID obtained from the NP information or based on local configuration, the local NRF sends a discovery request across PLMNs, interacting with the NRF in the target PLMN to retrieve the NF/NF service instance(s) that matches the discovery criteria.

-	The local NRF may receive in the inter-PLMN discovery response one of the following:

-	NF/NF service instance(s) matching the discovery criteria in the target PLMN.

-	The PLMN ID of the target PLMN as a result of ENUM and NP query in a transit PLMN, if applicable. The local NRF will then send an inter-PLMN discovery request to the NRF in the target PLMN if there is an interconnection agreement based on SBI.

-	No NF instance(s) found in the target PLMN, implying that SBI interactions should not be used.

-	The same behaviour described for the local NRF applies to the NRF in the target and transit PLMN.

-	If the local NRF finds NF/NF service instances for the MSISDN in the local PLMN or receives NF instances from the NRF in the target PLMN, the local NRF provides the NF instances in the discovery response for the NF consumer to send the next operation request in the procedure using SBI.

-	Otherwise, i.e. if there are not NF instances available that can serve the request, the local NRF provides the discovery response indicating the consumer NF to use a legacy interface for the next operation request in the procedure.

-	Based on discovery response from the local NRF, the NF service consumer (e.g. SMS-GMSC) sends the operation request using SBI to the NF service producer (e.g. UDM) or via legacy interface (e.g. MAP request to the HLR).

### 6.3.3	Procedures

The following sections illustrates the procedure to determine the target PLMN and the interface to be used for different scenarios depending on the PLMN the SMS recipient's MSISDN belongs to and NP requirement.

#### 6.3.3.1	Scenario 1: Selection of target PLMN based on MSISDN belonging to local PLMN

Figure 6.3.3.1 shows the procedure to select the target PLMN when the local NRF performs ENUM query and the MSISDN belongs to the local PLMN, i.e. the same PLMN of the NF service consumer (e.g. SMS-GMSC).

The discovery request to the local NRF (step 2) includes the optional indication for the NRF to determine the target PLMN and interface to be used.

In this case, when the ENUM response indicates to the local NRF that the MSISDN belongs to the local PLMN and SBI can be used, the local NRF performs a local search of NF instances that are provided in the discovery response (steps 4a-7a).

Figure 6.3.3.1 Selection of target PLMN based on MSISDN belonging to the local PLMN

#### 6.3.3.2	Scenario 2: Selection of target PLMN based on MSISDN belonging to a different PLMN with NP

Figure 6.3.3.2 shows the procedure to select the target PLMN when the MSISDN belongs to a different PLMN from the PLMN of the NF service consumer (e.g. SMS-GMSC), and Number Portability service is applicable.

In this case, the DNS/ENUM server is configured to perform an NP query and the response is used by the local NRF to obtain the target PLMN ID (steps 4-8). After the inter-PLMN discovery request, which includes the optional indication to determine the interface and the indication that NP service has already been invoked, the target PLMN determines the interface to be used (steps 9a-13a).

Figure 6.3.3.2 Selection of target PLMN based on MSISDN belonging to a different PLMN with NP

#### 6.3.3.3	Scenario 3: Selection of target PLMN based on MSISDN belonging to a different PLMN without NP

Figure 6.3.3.3 shows the procedure to select the target PLMN when the NRF performs ENUM query, the MSISDN belongs to a different PLMN from the PLMN of the NF service consumer (e.g. SMS-GMSC), and Number Portability service is not applicable. This is the case of a target PLMN in a different country or in the same country where NP is not required.

In this case, the ENUM response indicates to the local NRF that the MSISDN belongs to a different PLMN including a URI pointing to the NRF in the target PLMN (step 4a). After the inter-PLMN discovery request including the optional indication, the target PLMN determines the interface to be used (steps 6a-10a).

Figure 6.3.3.3 Selection of target PLMN based on MSISDN belonging to a different PLMN without NP

#### 6.3.3.4	Scenario 4: Selection of target PLMN based on MSISDN belonging to a different PLMN via transit PLMN

Figure 6.3.3.4 shows the procedure to select the target PLMN when the MSISDN belongs to a different PLMN from the PLMN of the NF service consumer (e.g. SMS-GMSC), and the target PLMN is determined via a transit PLMN. This may be the case of a target PLMN in a different country where NP query is required and performed in a transit PLMN.

In this case, the ENUM response indicates to the local NRF that the MSISDN belongs to a different PLMN including a URI pointing to the NRF in the transit PLMN (step 4). After the inter-PLMN discovery request including the optional indication, the NRF in the transit PLMN performs ENUM and subsequent NP queries, maps the information received to the target PLMN ID and includes the target PLMN ID in the inter-PLMN discovery response (steps 7-10), so that the local NRF can check whether SBI interconnection exists between local and target PLMNs.

Figure 6.3.3.4 Selection of target PLMN based on MSISDN belonging to a different PLMN via transit PLMN

### 6.3.4	Impacts on services, entities and interfaces

SMS-GMSC:-	Support service discovery request based on GPSI using NRF, including the indication to determine the target PLMN and interface to be used for the next operation in the procedure.

-	Support SBI interface to UDM.

NRF:

-	Support service discovery request based on GPSI including the indication to determine the target PLMN and interface to be used for the next operation in the procedure.

-	Support ENUM query, including information provided by NP service.

-	Determine target PLMN and interface to be used based on information received from ENUM/NP, or the inter-PLMN discovery response or local configuration.

ENUM:

-	Existing Enumservice type/subtype may be reused or new one(s) may be specified to be used for 5GC/SBA.

## 6.4	Solution#4: MT SM transfer

### 6.4.1	Introduction

This solution is to address Key Issue #1 "SBI-based SM MT message transfer". Introduce the solution for successful SM MT message transfer based on SBI in 5GC.

Figure 6.4.1-1 (copied from TS 3GPP 23.502 [4] clause 4.13.3.6) below is the baseline procedure of MT SM over NAS from TS 3GPP 23.502 [4]. We can observe that the interfaces between SMSF and SMS-GMSC, SMS-GMSC and UDM are still based on legacy protocol. This solution targets to provide the new SBI-based interfaces between the SMSF and the SMS-GMSC, the SMS-GMSC and the UDM, and route based on SBI for successful MT SM transfer including transfer of MT SM and transfer of the report for MT SM.

Figure 6.4.1-1: MT SM over NAS in CM_IDLE state via 3GPP access

### 6.4.2	Transfer of MT SM based on SBI

On the path to transfer MT SM, the SMSF needs add the new service or service operation based on SBI which SMS-GMSC shall invoke to deliver the MT SM to the SMSF. The SMSF has already provided SBI-based service Nsmsf_SMService to activate/deactivate the SMS service and to send uplink SMS for UE, a new service operation DownlinkSMS shall add in this service in order to send the downlink SM to the UE, and this service operation is defined in the Figure 6.4.2-1 below.

Figure 6.4.2-1: Send MT SM payload in downlink direction

1.	The NF Service Consumer (e.g. SMS-SGMS) shall send a POST request to the resource representing the UEContext (i.e. …/ue-contexts/{supi}/sendsms-dl) of the SMSF. The payload body of the POST request shall contain the SMS record to be sent, Service Centre address, callbackURI, and optionally contains SM Delivery Timer and SM Delivery Start Time if the UE is using extended idle mode DRX.

2a.	On success, "200 OK" shall be returned with "SmsRecordDeliveryDataDl" object in the response body, the payload body of the POST response shall contain the status of SMS record delivery attempts at the SMSF.

2b.	On success, "202 Accepted" shall be returned to indicate that the sending request has been accepted and MT SM record has been buffered in SMSF if the UE is using extended idle mode DRX and the UE is expected to respond to paging shortly or within the time frame indicated by the SM Delivery Timer and SM Delivery Start Time in request body, and the SMSF should reattempt to deliver the short message to the UE within this time frame.

2c.	On failure or redirection, the appropriate HTTP status code (e.g. "403 Forbidden") indicating the error shall be returned.

### 6.4.3	Routing MT SM based on SBI

Before SMS-GMSC sends the MT SM to serving SMSF, SMS-GMSC need to get the address or FQDN of serving SMSF node for UE. In order to get address or FQDN of serving SMSF node for UE, SMS-GMSC discovers and selects the UDM that manages the user subscriptions first, and then queries the serving SMSF node for UE from the selected UDM, finally SMS-GMSC invokes the Nnrf_NFDiscovery Service to get the address or FQDN of serving SMSF node for UE, the basic procedure is shown in Figure 6.z.3-1 below.

Figure 6.4.3-1: Routing MT SM in downlink direction

0.	UDM registers the NF profile an NF service profiles including the supported SUPI ranges and/or GPSI ranges in the NRF. And SMSF also registers NF profile an NF service profiles including IP addresses and/or FQDNs of the NF in the NRF.

1.	SC send the MT SM to SMS-GMSC.

2.	SMS-GMSC invokes the Nnrf_NFDiscovery_NFDiscovery to get the UDM instance that manages the user subscriptions using the GPSI/SUPI.

3.	SMS-GMSC invokes Nudm_SmsRoutingInfo_Get to get the serving SMSF instance for UE from the selected UDM.

4.	SMS-GMSC invokes the Nnrf_NFDiscovery_NFDiscovery to get the IP addresses or FQDNs of the serving SMSF for UE.

5.	SMS-GMSC sends the MT SM records to the SMSF.

Based on the solution above, the existing 5GC system need to be enhanced as below:

-	NRF is enhanced to support SMSF register and discover.

-	SMSF is enhanced to register itself in the NRF.

-	UDM is enhanced to support SMS-GMSC invokes Nudm_SmsRoutingInfo_Get to get the serving routing information for SM for UE.

### 6.4.4	Transfer of Report to MT SM based on SBI

SMSF sends the report to MT SM using notification mechanism, callbackURI is carried in the request message of service operation DownlinkSMS (see the definition of the service operation in clause 6.z.2) when SMS-GMSC sends the MT SM to SMSF. SMSF adds Nsmsf_SMService ReportNotify service operation to notify SMS-GMSC the report using the previously received callbackURI. The Figure 6.4.4-1below shows the service operation of notification.

Figure 6.4.4-1: Notify the Report to MT SM

1.	The SMSF sends a POST request to the callbackURI as provided by the NF service consumer (e.g. SMS-GMSC) during Sending MT SM payload. The request body shall contain the Report data to original MT SM.

2.	The NF service consumer responds with "204 No Content".

### 6.4.5	Impacts on services, entities and interfaces

SMSF：

-	Support new Nsmsf_SMService DownlinkSMS service operation

-	Support registering the SMSF NF profile and SMSF NF service profile in NRF

-	Support new Nsmsf_SMService ReportNotify service operation

SMS-GMSC:

-	Support invoking Nnrf_NFDiscovery to discover UDM using the User's identifier and Select the UDM

-	Support invoking Nudm_SmsRoutingInfo_Get to get the serving routing information for SMS and then invoking Nnrf_NFDiscovery to get the IP addresses or FQDNs of serving SMSF

-	Support invoking Nsmsf_SMService DownlinkSMS to send MT SM

-	Support receiving Nsmsf_SMService ReportNotify serivice operation

UDM:

-	Support Nudm_ SmsRoutingInfo service to provide routing information for SMS.

NRF

-	Support the registration and discovery of SMSF NF profile and NF service profile.

## 6.5	Solution#5: MO SM transfer

### 6.5.1	Introduction

This solution is to address Key Issue #2 "SBI-based SM MO message transfer". Introduce the solution for successful SM MO message transfer based on SBI in 5GC.

Figure 6.5.1-1 (from 3GPP TS 23.502 [4] clause 4.13.3.3) below is the baseline procedure for MO SM over NAS in 3GPP TS 23.502 [4]. We can observe that the interfaces between the SMSF and the SMS-IWMSC, the SMS-IWMSC and the UDM are still based on legacy protocol. This solution targets to provide the new SBI-based interfaces between the SMSF and the SMS-IWMSC, the SMS-IWMSC and the UDM, and route based on SBI for successful MO SM transfer including transfer of MO SM message and the transfer of the report of MO SM.

Figure 6.5.1-1: MO SM over NAS

This solution will cover the scenarios below:

-	The requirement of that the SMSF gets the routing information to SMS-IWMSC based on SBI needs be addressed

-	MO SM transfer between SMSF and SMS-IWMSC based on SBI

-	MO SM report transfer between SMSF and SMS-IWMSC based on SBI

### 6.5.2	SMS-IWMSC Discovery and Selection

More than one SMS-IWMSC may be deployed in the HPLMN (in the operator's network), and these SMS-IWMSCs may be deployed based on the serving location, or based on the serving UE identity ranges (e.g. SUPI ranges, GPSI ranges etc.), or based on the connected E.164 address of the SC. The SMS-IWMSC registers the routing information with other NF or NF service information together in the NRF according to the specific deployment. The SMSF queries the NRF to get the routing information related to the SMS-IWMSC instance or the SMS-IWMSC service instance.

For supporting different deployments of the SMS-IWMSC instances, the following factors may be registered in NRF in order to support discovery of the SMS-IWMSC instances or the SMS-IWMSC service instances by the SMSF according to the discovery solution mentioned in this clause:

-	supported SUPI ranges.

-	supported GPSI ranges

-	supported location (e.g. TAIs, CGIs, etc.)

-	supported E.164 address of the SC

Accordingly the following factors shall be supported as query parameters for discovering the proper SMS-IWMSC instances or SMS-IWMSC by the SMSF.

-	SUPI

-	GPSI

-	Location of UE (e.g. TAIs, or CGIs etc.).

-	E.164 address of the SC

The IP addresses or FQDN of the SMS-IWMSC instances or the SMS-IWMSC service instances should be returned to the SMSF, and the SMSF shall route the MO SM to the SMS-IWMSC instances or SMS-IWMSC service instances based on the IP addresses or FQDN of the SMS-IWMSC instances or SMS-IWMSC service instances.

### 6.5.3	Transfer of MO SM based on SBI

The SMSF has already provided the support of Nsmsf_SMService UplinkSM service operation to support uplink SM transfer, see 3GPP TS 29.540 [6] clause 5.2.2.4.

SMS-IWMSC is still based on legacy interface. The interface based on SBI need to be defined, therefore the SMS-IWMSC adds a new Niwmsc_SMS service and SMSSubmit service operation to support the transfer of the MO SM based on SBI, and Niwmsc_SMS_SMSSubmit service operation is defined as shown in Figure 6.b.3-1 below.

Figure 6.5.3-1: Send MO SM payload in downlink direction

1.	The NF Service Consumer (e.g. SMSF) shall send a POST request to the resource URI associated with the "mosms" custom operation. The payload body of the POST request shall contain the SM record to be sent, the Service Centre address, the callbackURI for report to MO SM, the timer for waiting the report to MO SM, and optionally contains the Access Type.

2a.	On success, "200 OK" shall be returned with "SmsRecordDeliveryDataMo" object in the response body, the payload body of the POST response shall contain the status of SM record delivery attempts at the SMS-IWMSC.

2b.	On failure or redirection, the appropriate HTTP status code (e.g. "403 Forbidden") indicating the error shall be returned.

### 6.5.4	Transfer of Report to MO SM based on SBI

SMS-IWMSC shall send the report for MO SM using notification mechanism, the callbackURI is carried in the request message of service operation UplinkSM (see the definition of the service operation in clause 6.5.3) when SMSF sends the MO SM to SMS-IWMSC. The SMS-IWMSC adds Niwmsc_SMS_ReportNotify service operation to notify SMSF the report using the previously received callbackURI. The Figure 6.5.4-1 below shows the service operation for notification.

Figure 6.5.4-1: Notify the Report to MO SM

1.	The SMSF sends a POST request to the callbackURI as provided by the NF service consumer (e.g. SMSF) during Sending MO SM payload. The request body shall contain the Report data to original MO SM.

2.	The NF service consumer responds with "204 No Content".

### 6.5.5	Impacts on services, entities and interfaces

SMS-IWMSC：

-	Support new Niwmsc_SMS service

-	Support registering the SMS-IWMSC NF profile and S SMS-IWMSC MSF NF service profile in NRF

SMSF:

-	Support invoking Nnrf_NFDiscovery to discover SMS-IWMSC in this solution

-	Support invoking Niwmsc_SMS to send MO SM and receive Report to MO SM

NRF

-	Support the registration and discovery of SMS-IWMSC NF profile and NF service profile.

## 6.6	Solution#6: MT SM transfer via SMS Router/IP-SM-GW

### 6.6.1	Introduction

This solution is to address Key Issue #1 "SBI-based SM MT message transfer". Current procedure of MT SM transfer via SMS Router/IP-SM-GW is shown in Figure 6.6.1-1, interfaces between SMS-GMSC and UDM, UDM/SMSF and SMS Router/IP-SM-GW are still based on legacy protocol. This solution introduces a successful SM MT message transfer based on SBI in 5GC with SMS Router/IP-SM-GW deployed, steps 2, 4 and 6 are changed to messages in SBI interfaces, and other steps will not be impacted.

Figure 6.6.1-1: Successful MT SM transfer via the SMS Router/IP-SM-GW

### 6.6.2	Routing MT SM via SMS Router/IP-SM-GW based on SBI

As shown in Figure 6.6.2-1, step 3 is identical to step 2 in Figure 6.6.1-1, step 5 is identical to step 4 in Figure 6.6.1-1 and step 6 is identical to step 6 in Figure 6.6.1-1. This procedure also defines the UDM/SMSF registration and discovery before sending the message.

Figure 6.6.2-1: Successful MT SM transfer via the SMS Router/IP-SM-GW based on SBI

0	The UDM registers the NF profile including the supported SUPI ranges and/or GPSI ranges in the NRF. The SMSF also registers NF profile including the IP addresses and/or the FQDNs of the NF in the NRF.

1	The SC sends the MT SM to the SMS-GMSC.

2	The SMS-GMSC invokes the Nnrf_NFDiscovery_NFDiscovery to retrieve the UDM instance that manages the user subscriptions using the GPSI/SUPI.

3a.-3d.	Alternative 1: the SMS-GMSC invokes Nudm_UECM_Get to retrieve the routing information of the UE to be able to send the MT SM from the selected UDM. The UDM invokes Nsmsrouter_SMService_Get or Nipsmgw_SMService_Get to retrieve the address of the SMS Router/IP-SM-GW to receive the MT SM, the UDM also provides the SMSF instance to the SMS Router/IP-SM-GW. The address of the SMS Router/IP-SM-GW to be contacted by the UDM may be configured locally. The SMS Router/IP-SM-GW provides address to the UDM to receive the MT SM. UDM provides the received address to the SMS-GMSC.

3e.-3j.	Alternative 2: the SMS-GMSC invokes Nudm_UECM_Get to retrieve the routing information of the UE to be able to send the MT SM from the selected UDM, UDM responses the message with 3xx redirection, the URI of the SMS Router/IP-SM-GW is included location header. After that, the SMS-GMSC invokes Nsmsrouter_SMService_Get or Nipsmgw_SMService_Get to retrieve the address of the SMS Router/IP-SM-GW to receive the MT SM, and the SMS Router/IP-SM-GW queries the UDM by invoking the Nudm_UECM_Get to retrieve the SMSF instance. The SMS Router/IP-SM-GW provides address to the SMS-GMSC to receive the MT SM.

4	The SMS-GMSC sends the MT SM to SMS Router/IP-SM-GW.

5	The SMS Router/IP-SM-GW invokes the Nnrf_NFDiscovery_NFDiscovery to retrieve the IP addresses or FQDNs of the serving SMSF of the UE based on the SMSF instance received in step 3b.

6a.	The SMS Router/IP-SM-GW sends the MT SM records to the SMSF.

6b and 6c.	The SMSF responses SMS Router/IP-SM-GW for the receiving of the MT SM records and sends delivery report to the SMS Router/IP-SM-GW later – alternative 1.

6c.	The SMSF responses SMS Router/IP-SM-GW with the delivery report included – alternative 2.

### 6.6.3	Get routing information from SMS Router/IP-SM-GW

The SMS Router/IP-SM-GW shall support the new service (SMService) and service operation (Get) based on SBI, which may be invoked by UDM (step 3b, described in clause 6.6.2) or the SMS-GMSC (step 3g, described in clause 6.6.2) to get the routing information as defined in Figure 6.6.3-1 below.

Figure 6.6.3-1: Get routing information

1.	The NF Service Consumer (e.g. UDM or SMS-GMSC) shall send a GET request to the resource representing the UE's routing information, if the NF Service Consumer is UDM, the query parameters includes the NF Instance ID of the SMSF.

2a.	On success, the SMS Router/IP-SM-GW responds with "200 OK" with the message body containing the UE's routing information to receive the MT SM.

2b.	On failure or redirection, the appropriate HTTP status code (e.g. "403 Forbidden") indicating the error shall be returned.

### 6.6.4	Transfer of MT SM from SMS Router/IP-SM-GW to SMSF

The SMSF shall support the new service or service operation based on SBI, which may be invoked by SMS Router/IP-SM-GW to deliver the MT SM to SMSF. A new service operation DownlinkSMS may be extended to the service Nsmsf_SMService supported by the SMSF as defined in Figure 6.6.4-1 below.

Figure 6.6.4-1: Transfer of MT SM from SMS Router/IP-SM-GW to SMSF

1.	The NF Service Consumer (e.g. SMS Router/IP-SM-GW) shall send a POST request to the resource representing the UEContext (i.e. …/ue-contexts/{supi}/sendsms-dl) of the SMSF. The payload body of the POST request shall contain the SMS record to be sent, Service Centre address, callbackURI of the SMS Router/IP-SM-GW, and optionally contains the SM Delivery Timer and SM Delivery Start Time if the UE is using extended idle mode DRX. For alternative 2 described in step 6d of clause 6.6.2, the callbackURI of the SMS Router/IP-SM-GW is not needed.

2a.	On success, "200 OK" shall be returned with "SmsRecordDeliveryDataDl" object in the response body, the payload body of the POST response shall contain the status of SMS record delivery attempts at the SMSF. For alternative 2 described in step 6d of clause 6.6.2, the Report data of the original MT SM is included in the response body.

2b.	On success, "202 Accepted" shall be returned to indicate that the sending request has been accepted and MT SMS record has been buffered in SMSF if the UE is using extended idle mode DRX and the UE is expected to respond to paging shortly or within the time frame indicated by the SM Delivery Timer and SM Delivery Start Time in request body, and the SMSF should reattempt to deliver the short message to the UE within this time frame. For alternative 2 described in step 6d of clause 6.6.2, this step is not needed.

2c.	On failure or redirection, the appropriate HTTP status code (e.g. "403 Forbidden") indicating the error shall be returned.

### 6.6.5	Transfer of Report from SMSF to SMS Router/IP-SM-GW

This procedure is only applicable to alternative 1 described in step 6c of clause 6.6.2.Based on the callback URI of the SMS Router/IP-SM-GW received, the SMSF sends notification to the SMS Router/IP-SM-GW to notify the delivery report of MT SM. The SMSF needs to support the new service operation (Notify) in Nsmsf_SMService service. The Figure 6.6.5-1 below shows the service operation of notification.

Figure 6.6.5-1: Transfer of Report from SMSF to SMS Router/IP-SM-GW

1.	The SMSF sends a POST request to the callbackURI as provided by the NF service consumer (e.g. SMS Router/IP-SM-GW) during transferring of MT SM from the SMS Router/IP-SM-GW to the SMSF as defined in clause 6.6.4. The request body shall contain the Report data of the original MT SM.

2.	The NF service consumer responds with "204 No Content".

### 6.6.6	Impacts on services, entities and interfaces

SMSF：

-	Support new Nsmsf_SMService DownlinkSMS service operation, invoked by SMS Router/IP-SM-GW.

-	Support new Nsmsf_SMService Notify service operation to send the notification to SMS Router/IP-SM-GW.

SMS Router/IP-SM-GW:

-	Support new service Nsmsrouter_SMService or Nipsmgw_SMService and the Get service operation, invoked by UDM or the SMS-GMSC.

UDM:

-	Update the Nudm_UECM_Get, invoked by SMS-GMSC to retrieve the routing information of SMS Router/IP-SM-GW.

-	Update the Nudm_UECM_Get, invoked by SMS Router/IP-SM-GW to retrieve the routing information of SMSF.

## 6.7	Solution#7: Unsuccessful MT SM transfer via SMS Router/IP-SM-GW

### 6.7.1	Introduction

This solution is to address Key Issue #1 "SBI-based SM MT message transfer". Introduce the solution on unsuccessful SM MT message transfer via SMS Router/IP-SM-GW based on SBI in 5GC.

### 6.7.2	Failure for MT SMS transfer in SMS Router/IP-SM-GW

The UDM may invoke the Nsmsrouter_SMService_Get or Nipsmgw_SMService_Get from the SMS Router/IP-SM-GW to get the routing information for MT SM.

The SMS Router/IP-SM-GW responds CONTEXT_NOT_FOUND with 404 Not Found to the UDM if no routing information is available for this UE.

The SMS Router/IP-SM-GW responds NETWORK_FAILURE with 504 Gateway Timeout to the UDM if the request is rejected due to a network problem.

### 6.7.3	Impacts on services, entities and interfaces

SMS Router/IP-SM-GW:

-	Support of the failure cases in the new service Nsmsrouter_SMService or Nipsmgw_SMService and the Get service operation.

## 6.8	Solution#8: Unsuccessful MT SM transfer

### 6.8.1	Introduction

This solution is to address Key Issue #1 "SBI-based SM MT message transfer". Introduce the solution for unsuccessful SM MT message transfer based on SBI in 5GC.

Unsuccessful message transfer from the SC to the UE may be caused by a variety of different errors. These errors may happen in the UE (e.g. Memory Capacity Exceeded), or may happen in the network (e.g. NF overload, link error between NFs), some of them may be temporary or some of them may be permanent. An error indication shall be return to the SC from the SMS-GMSC in order that the SC knows how to handle next, discard or reattempt. And for some errors which can be recovered in short time, the network should provide ways that the SC can be aware of the recovery and reattempt the send at a proper time.

The error indication related to mobile terminated short message transfer which may be transferred to the originating SC  see 3GPP TS 23.040 [2] clause 3.3.2 Table 1. This clause provides the solutions of Error indications based on SBI between the SMSF and the SMS-GMSC, and the UDM and the SMS-GMSC when errors happen during MT SM transfer, and how to map these Errors indication to the Errors indication defined in 3GPP TS 23.040 [2] clause 3.3.2 Table 1.

### 6.8.2	Failure for MT SM transfer in SMSF

SMSF adds new service operation DownlinkSMS in existing Nsmsf_SMService to transfer MT SM (see the description of the new service operation in solution 4 MT SM transfer of clause 6.4). SMSF indicates the error for MT SM transfer in DownlinkSMS response if the transfer failed. The Application Errors used in DownlinkSMS response are defined in Table 6.8.2-1 below, the SMS-GMSC shall map the Application Error listed in Table 6.8.2-1 to the Error indications which has been defined in 3GPP TS 23.040 [2] clause 3.3.2 Table 1 when SMS-GMSC receives them, and the other Application Errors not listed are suggested to be mapped to "System failure" see 3GPP TS 23.040 [2] clause 3.3.2 Table 1.

Table 6.8.2-1: Application errors

| Application Error | HTTP status code | Description | Error indication Mapped to |
| --- | --- | --- | --- |
| USER_NOT_FOUND | 404 Not Found | The provided subscriber identifier is invalid or the service user not found from UDM. | Unknown subscriber |
| CONTEXT_NOT_FOUND | 404 Not Found | The UE context for SMS is invalid or not found in SMSF. | Unknown subscriber |
| SERVICE_NOT_ALLOWED | 403 Forbidden | The requested service is barred for this service user (e.g. Barring of all MT-SMS). | Call barred |
| USER_UNREACHABLE | 403 Forbidden | The delivery of the short message to UE failed due to UE is not reachable | Absent subscriber |
| UE_DELIVERY_FAILURE | 403 Forbidden | The delivery of the short message to UE failed due to failure in UE at reception of a short message, e.g. protocol error. | Error in MS |
| MEMORY_CAPACITY_EXCEEDED | 403 Forbidden | The UE rejects the short message since it has no memory capacity available to store the message. | Memory Capacity Exceeded |
| HIGHER_PRIORITY_REQUEST_ONGOING | 409 Conflict | The delivery of MT SM failed due to a higher priority session ongoing and SMSF can't buffer the MT SM. This failure may happen because SMSF receive the "HIGHER_PRIORITY_REQUEST_ONGOING" with 409 Conflict from AMF. | MS busy for MT SMS |
| TEMPORARY_REJECT_REGISTRATION_ONGOING | 409 Conflict | The delivery of MT SM failed due to an ongoing registration procedure and SMSF can't buffer the MT SM. This failure may happen because SMSF receive the "HIGHER_PRIORITY_REQUEST_ONGOING" with 409 Conflict from AMF. | MS busy for MT SMS |
| TEMPORARY_REJECT_HANDOVER_ONGOING | 409 Conflict | The delivery of MT SM failed an ongoing N2 handover procedure and SMSF can't buffer the MT SM. This failure may happen because SMSF receive the "HIGHER_PRIORITY_REQUEST_ONGOING" with 409 Conflict from AMF. | MS busy for MT SMS |
| SMS_PAYLOAD_MISSING | 400 Bad Request | The expected SMS payload content is missing. |  |
| SMS_PAYLOAD_ERROR | 400 Bad Request | Errors exist in the format of SMS payload. |  |

Editor's Node:	It need be further studied about how to further harmonize with 3GPP TS 29.002 12.9.3 MTFSMRes user errors.

If USER_UNREACHABLE with 403 Forbidden is returned in DownlinkSMS response, a structure the data UeAbsentError shall be returned in the response body, the UeAbsentError shall contain ProblemDetails, may contain AbsentUserDiagnosticUe (The values are defined in 3GPP TS 23.040[3] clause 3.3.2), and may contain the estimated maximum waiting time in seconds before the UE will be reachable (e.g. If the UE is in MICO mode or If the UE is using extended idle mode DRX, this time shall be received from AMF).

If UE_DELIVERY_FAILURE with 403 Forbidden is returned in DownlinkSMS response, a structure the data UeDeliveryError shall be returned in the response body, the UeDeliveryError shall contain ProblemDetails and contain UeDelvieryFailureCause, UeDelvieryFailureCause shall contain complementary information associated to UE_DELIVERY_FAILURE.

If the SMSF can't transfer the MT SM within the time frame indicated by the SMS-GMSC (e.g. indicated in SM-Delivery-Timer and SM-Delivery-Start-Time) and the SMSF can buffer the short message record, the SMSF shall respond the DownlinkSMS request with 202 Accepted, and notify the SMS-GMSC the result of the MT SM delivery using the CallBackUri provided by the SMS-GMSC in the DownlinkSMS request when the time frame indicated by SMS-GMSC previously expires. The notification uses newly added Nsmsf_SMService TransferFailureNotify service operation if the MT SM delivery failed. The Figure 6.a.2-1below shows the service operation for notification.

Figure 6.8.2-1: MT SM Transfer Failure Notify

1.	When the SMSF determines that the delivery of MT SM has failed, the SMSF sends a POST request to the callbackURI as previously provided by the NF service consumer (e.g. SMS-GMSC) during Sending MT SM payload. The request body shall contain the MT SM Transfer Failure Data to include the failure cause and resource URI returned in the Location header when the DownlinkSMS was initiated. The failure cause shall be set to one of following cause value:

-	USER_UNREACHABLE

-	HIGHER_PRIORITY_REQUEST_ONGOING

-	TEMPORARY_REJECT_REGISTRATION_ONGOING

-	TEMPORARY_REJECT_HANDOVER_ONGOING

2.	The NF service consumer responds with "204 No Content".

### 6.8.3	Failure for MT SM transfer in UDM

Before the SMS-GMSC transfers the MT SM to the SMSF, the SMSF shall retrieve the serving SMSF information from the UDM using the existing Nudm_UEContextManagement_Get service operation.

If the UDM responds USER_NOT_FOUND with 404 Not Found to the SMS-GMSC, the SMS-GMSC shall map it to "Unknown subscriber" and return this Error indication to the SC.

If the UDM responds CONTEXT_NOT_FOUND with 404 Not Found to the SMS-GMSC, the SMS-GMSC shall query SMS Subscription Data using existing Nudm_SubscriberDataManagement SMS Subscription Data Retrieval service operation, if it indicates that the UE subscription doesn't allow SMS delivery over NAS in the SMS Subscription Data, the SMS-GMSC shall indicates Error indication "Teleservice not provisioned" to the SC, or if it indicates that the UE subscription allows SMS delivery over NAS in the SMS Subscription Data, the SMS-GMSC shall indicates Error indication "Absent subscriber" to the SC.

### 6.8.4	Impacts on services, entities and interfaces

SMSF：

-	Support new Nsmsf_SMService DownlinkSMS service operation with the new Error Indication.

-	Support new Nsmsf_SMService TransferFailureNotify service operation

SMS-GMSC:

-	Support invoking new Nsmsf_SMService DownlinkSMS service operation with the new Error Indication, and corresponding operations in SMS-GMSC.

-	Support receiving Nsmsf_SMService TransferFailureNotify service operation and corresponding operations in the SMS-GMSC

-	Support invoking Nudm_UEContextManagement_Get and Nudm_SubscriberDataManagement SMS Subscription Data Retrieval service operation, and the corresponding operations in the SMS-GMSC

## 6.9	Solution#9: Alert

### 6.9.1	Introduction

This solution is to address Key Issue #1 "SBI-based SM MT message transfer". The SM MT message transfer failed because of some reasons like UE was unreachable or the memory capacity for SMS was exceeded, the Alert-SC service provides the service on that the network informs the SC that an UE,

1)	to which a delivery attempt has failed because the UE was not reachable or because the UE memory capacity was exceeded; and

2)	which is now recognized by the PLMN:

a)	to have resumed operation (e.g. to have responded to a paging request); or

b)	to have memory newly available (which implies that the mobile is reachable).

is again ready to receive one or more short messages. The SC may initiate the delivery attempt procedure for the queued messages destined for this UE on reception of an Alert-SC.

The solution in this clause covers,

1)	How to request Alert and how to transfer Alert if UE SM MT message transfer failed because UE was unreachable and this MT SM for the UE requested to be retransmitted at a later time,

2)	How to request Alert and how to transfer Alert if UE SM MT message transfer failed because the memory capacity for SMS was exceeded and this MT SM for the UE requested to be retransmitted at a later time.

The service operations between SMS-GMSC and SMSF, SMS-GMSC and UDM, UDM and SMSF, UDM and AMF based on SBI are shown in following Figure 6.9.1-1.

Figure 6.9.1-1: The alert procedure

1	When the SMSF determines that the delivery of MT SM has failed, the SMSF sends a POST request using the callbackURI as previously provided by the SMS-GMSC.

2	The SMS-GMSC notifies the Failure Report to the SC.

3	If the cause of failure is because of the UE is unreachable or the UE memory capacity is exceeded, the SMS-GMSC sends Nudm_EE_Subscribe to the UDM to subscribe to the event "UE_REACHABILITY_FOR_SMS" or "UE_MEMORY_AVAILABLE_FOR_SMS".

4a	If the UDM receives the subscription to "UE_MEMORY_AVAILABLE_FOR_SMS", the UDM sends the Nsmsf_EE_subscribe request to SMSF to subscribe to the notification on that UE have memory newly available for SMS.

4b	If the UDM receives the subscription to "UE_REACHABILITY_FOR_SMS" and the UDM hasn't subscribed to the UE available event on AMF, the UDM sends the Namf_EE_subscribe request to the AMF to subscribe to the notification on reachability of that UE.

5a	If SMSF receives the indication on UE has memory newly available for SMS, the SMSF notifies the UDM of the event.

5b	If the UE is reachable, the AMF notifies the UDM of that event.

6	The UDM notifies the "UE_MEMORY_AVAILABLE_FOR_SMS" or "UE_REACHABILITY_FOR_SMS" to the SMS-GMSC accordingly.

7	The SMS-GMSC alerts the SC.

The following clauses are to introduce the services based on SBI related to "UE_MEMORY_AVAILABLE_FOR_SMS" or "UE_REACHABILITY_FOR_SMS" separately.

### 6.9.2	Alert MS memory capacity available

When the SMS-GMSC receives the failure that the UE memory for SMS is exceeded and the SMS-GMSC decided reattempting the delivery of the MT SM, the SMS-GMSC subscribes to the event "UE_MEMORY_AVAILABLE_FOR_SMS" provided by the network, and the network shall notify the SMS-GMSC when the event happened.

To support the subscription and notification of the event "UE_MEMORY_AVAILABLE_FOR_SMS", the existing UDM EE subscribe and Notify service operation shall be extended to support the event. See Subscription to Notification of event occurrence service operation of UDM EE service in 3GPP TS 29.503 [7] clause 5.5.2.2.2, the specific and extended part of the service operation is described below:

In the subscription request, the event type is set to the newly added "UE_MEMORY_AVAILABLE_FOR_SMS", the expiry time is set to the maximum store time of MT SM in SC, the MaxNumOfReports is set to 1, and Monitoring Configuration data includes the SC address.

Event Occurrence Notification service operation of UDM EE service is described in 3GPP TS 29.503 [7] clause 5.5.2.4.2, the extended part of the service operation is that eventType indicates "UE_MEMORY_AVAILABLE_FOR_SMS" and the SC address is also included in the notify request.

To support the subscription and notification of the event "UE_MEMORY_AVAILABLE_FOR_SMS", the SMSF added a new service Nsmsf_EventExpoure, and subscribe/unsubscribe/notify service operation on this new service.

When the UDM receives the EE subscription to event "UE_MEMORY_AVAILABLE_FOR_SMS", the UDM shall invoke the Nsmsf_EventExpoure_subscribe to subscribe to the event "UE_MEMORY_AVAILABLE_FOR_SMS" in the SMSF, see Figure 6.9.2-1.

Figure 6.9.2-1: NF service consumer subscribes to notifications

1.	The NF service consumer (e.g. UDM) sends a POST request to the parent resource (collection of subscriptions) (.../{ueIdentity}/ee-subscriptions), to create a subscription as present in message body. The request may event type (e.g. mobile has memory available), the callbackURI to which the notification will send, the configuration related to the Event (e.g. one time event).

2a.	On success, the UDM responds with "201 Created" with the message body containing a representation of the created subscription. The Location HTTP header shall contain the URI of the created subscription. If the event subscription was for a group of UEs:

When the UDM decided to unsubscribe to the event "UE_MEMORY_AVAILABLE_FOR_SMS" for which it has subscribed previously, the UDM shall invoke the Nsmsf_EventExpoure_unsubscribe to unsubscribe to the event "UE_MEMORY_AVAILABLE_FOR_SMS" in SMSF, see Figure 6.9.2-2.

Figure 6.9.2-2: NF service consumer unsubscribes to notifications

1.	The NF service consumer sends a DELETE request to the resource identified by the URI previously received during subscription creation.

2a.	On success, the UDM responds with "204 No Content".

2b.	If there is no valid subscription available (e.g. due to an unknown SubscriptionId value), HTTP status code "404 Not Found" shall be returned including additional error information in the response body (in the "ProblemDetails" element).

When SMSF received the RP-SMMA message from UE (see 3GPP 24.011 [8] clause 7.3.2), the SMSF is aware of memory available of the MS, then SMSF notifies the memory available to the consumers (e.g. UDM) which subscribes to this service. It is suggested that the SMSF adds a new service Nsmsf_EventExpoure to provide subscribe/unsubscribe/notify the events related the UE (e.g. mobile has memory available). The service operation are show below.

Figure 6.9.2-3: Event Occurrence Notification

1.	The UDM sends a POST request to the callbackURI provided by the NF service consumer when subscribing to the event, the request shall include the report related to the monitored event (e.g. mobile has memory available).

2.	The NF Service Consumer responds with "204 No Content".

### 6.9.3	Alert MS reachable

The UDM has already provided the support of the subscribe/unsubscribe/notify of the event UE_REACHABILITY_FOR_SMS by Nudm_EventExposure Service. If during the delivery of the MT SM to UE, the SMS-GMSC/SC receives that MT SM delivery failed and the cause of failure is unreachable for UE from SMSF, the SMS-GMSC/SC invokes Nudm_EventExposure Service to subscribe to the event UE_REACHABILITY_FOR_SMS, the Nudm_EventExposure_subscribe is extended to include the SC address in the request, and the UDM shall notify the SMS-GMSC that the UE is reachable for SMS using Nudm_EventExposure_Notify Service operation and callbackUri previously provided in the subscription request.

### 6.9.4	Impacts on services, entities and interfaces

SMS-GMSC:

-	Support invoking Nudm_EE service for "UE_MEMORY_AVAILABLE_FOR_SMS" and"UE_REACHABILITY_FOR_SMS" events.

-	Support alert the SC according to the received notification of "UE_MEMORY_AVAILABLE_FOR_SMS" and"UE_REACHABILITY_FOR_SMS" events.

UDM:

-	Extend the EE service to support subscribe/unsubscribe/notify new event "UE_MEMORY_AVAILABLE_FOR_SMS".

-	Support invoking Nsmsf_EventExpoure service.

SMSF:

-	Support newly added service Nsmsf_EventExpoure.

-	Support to register the newly added service Nsmsf_EventExpoure service in NRF.

NRF:

-	Support registration and discovery of the new defined services.

## 6.10	Solution#10: Alert the recovery of UE memory for SMS

### 6.10.1	Introduction

This solution is to address Key Issue #1 "SBI-based SM MT message transfer". The SM MT message transfer failed because of the memory capacity for SMS was exceeded, the Alert-SC service in the network informs the SC that an UE has memory newly available (which implies that the mobile is reachable) if the SMS-GMSC/SC decided reattempting the delivery and subscribed to the corresponding event in the network.

This solution covers how to request the Alert and how to transfer the Alert if the UE SM MT message transfer failed because the memory capacity for SMS was exceeded and this MT SMS for the UE requested to be retransmitted at a later time.The service operations between the SMS-GMSC and SMSF, SMS-GMSC and UDM, UDM and SMSF, UDM and AMF based on SBI are shown in following Figure 6.10.1-1.

Figure 6.10.1-1: The alert procedure

1.	The SMS-GMSC invokes Nsmsf_SMService_DownlinkSMS request to send the MT MS.

2.	When the SMSF determines that the delivery of MT SMS has failed, the SMSF invokes Nudm_UECM_Update to store the state of UE memory (i.e. UE memory exceeded).

3.	SMSF responds the SMS-GMSC with indication of the UE memory exceeded for the SMS in the Nsmsf_SMService_DownlinkSMS Response.

4.	The SMS-GMSC notify the Failure Report to the SC.

5.	The SMS-GMSC invokes Nudm_EE_Subscribe to subscribe to the event "UE_MEMORY_AVAILABLE_FOR_SMS".

6.	The AMF relay the RP-SMMA message from the UE to the SMSF using Namf_Communication N1 Message Notification service operation.

7.	The SMSF invokes Nudm_UECM_Update to update the state of UE memory (UE memory is available).

8.	The UDM notifies the "UE_MEMORY_AVAILABLE_FOR_SMS".

9.	The SMS-GMSC alerts the SC.

The first MT SM delivery failed from the same SC due to UE memory exceeded, the SMSF stores this UE state in UDM/UDR, the later MT SMs can query this state in UDM/UDR before delivery, and if UE state is UE Memory Exceeded, the SMS-GMSC/SC should/shall store the MT SM locally, and then subscribe to the event "UE_MEMORY_AVAILABLE_FOR_SMS" in the UDM, this way will avoid unnecessary signalling interactions between the SMS-GMSC/SC and the UE.

### 6.10.2	UE memory capacity state store

The UDM/UDR adds a new UE memory state for SMS update service operations in the Nudm_UEContextManagement Service to support that the SMSF provides the up to date state of UE memory capacity for SMS in the UDM/UDR. If the SMSF is aware of the change of UE memory capacity for SMS, the SMSF shall invoke this service operation to update the state of UE memory capacity.

Figure 6.10.2-1 and Figure 6.10.2-2 shows a scenario where the SMSF sends a request to the UDM/UDR to update the state of UE memory capacity within the SmsfRegistration resource. The request contains the UE's identity (/{ueId}) which shall be a SUPI and an instruction to modify a parameter (e.g. PEI).

Figure 6.10.2-1: UE memory state for SMS update for 3GPP access

1.	The AMF sends a PATCH request to the resource representing the UE's SMSF registration for 3GPP access. The Modification instruction contains the UE memory state data for SMS including the state of the UE memory for SMS (e.g. the memory for SMS exceeded or not), SC address, time of state change, expiry timer of the stored state.

2a.	On success, the UDM responds with "204 No Content".

2b.	If the resource does not exist e.g. the UE is not registered yet, HTTP status code "404 Not Found" should be returned including additional error information in the response body (in the "ProblemDetails" element).

2c.	If the resource exists, but the requesting SMSF is not the one currently registered for the UE, HTTP status code "422 Unprocessable Request" should be returned including additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the PATCH response body.

Figure 6.10.2-1: UE memory state for SMS update for Non 3GPP access

1.	The SMSF sends a PATCH request to the resource representing the UE's SMSF registration for Non 3GPP access. The Modification instruction contains the UE memory state data for SMS including the state of the UE memory for SMS (e.g. the memory for SMS exceeded or not), SC address, time of state change, expiry timer of the stored state.

2a.	On success, the UDM responds with "204 No Content".

2b.	If the resource does not exist e.g. the UE is not registered yet, HTTP status code "404 Not Found" should be returned including additional error information in the response body (in the "ProblemDetails" element).

2c.	If the resource exists, but the requesting SMSF is not the one currently registered for the UE, HTTP status code "422 Unprocessable Request" should be returned including additional error information in the response body (in the "ProblemDetails" element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the PATCH response body.

### 6.10.3	UE memory capacity state query

Extend the SMSF Registration information to include the UE memory capacity state information, and the UE memory capacity state information may contain the state of the UE memory for SMS (e.g. the memory for SMS exceeded or not), SC address, time of state change, expiry timer of the stored state. The service consumer (e.g. SMSF, SMS-GMSC) may invoke Nudm_UEContextManagement_Get to query the SMSF Registration information for 3GPP and Non 3GPP, if the UE memory capacity state information is absent in the SMSF Registration information, it implicitly indicates that the UE memory capacity for SMS is available.

### 6.10.4	UE memory capacity available event

Extends the existing Nudm_EE service to support subscription/unsubscription/notification of new event "UE_MEMORY_AVAILABLE_FOR_SMS". When the UDM receives the change of state of the UE memory for SMS from "exceeded" to "available", the UDM shall notify the consumer (e.g. SMS-GMSC) on event "UE_MEMORY_AVAILABLE_FOR_SMS" if the consumer has subscribed to the event.

See Subscription to Notification of event occurrence service operation of UDM EE service in 3GPP TS 29.503 [7] clause 5.5.2.2.2, the specific and extended part of the service operation is below:

In the subscription request, the event type is set to the newly added "UE_MEMORY_AVAILABLE_FOR_SMS", the expiry time is set to the maximum store time of MT SM in SC, the MaxNumOfReports is set to 1, and Monitoring Configuration data includes the SC address.

See Event Occurrence Notification service operation of UDM EE service in 3GPP TS 29.503 [7] clause 5.5.2.4.2, the extended part of the service operation is that eventType indicates "UE_MEMORY_AVAILABLE_FOR_SMS" and the the SC address is also included in the notify request.

For support the subscription and notification of the event "UE_MEMORY_AVAILABLE_FOR_SMS", the SMSF added a new service Nsmsf_EventExpoure, and subscribe/unsubscribe/notify service operation on this new service.

### 6.10.5	Alert MS reachable

The UDM has already provided the subscribe/unsubscribe/notify of the event UE_REACHABILITY_FOR_SMS with the Nudm_EventExposure Service. If the SMS-GMSC/SC received that MT SM delivery failed (during the delivery of the MT SM to the UE) and the cause of failure is unreachable for UE from SMSF, the SMS-GMSC/SC invokes Nudm_EventExposure Service to subscribe to the event UE_REACHABILITY_FOR_SMS, the Nudm_EventExposure_subscribe is extended to include the SC address in the request. and the UDM shall notify the SMS-GMSC that the UE is reachable for SMS using Nudm_EventExposure_Notify Service operation and the callbackUri previously provided in the subscription request.

### 6.10.6	Impacts on services, entities and interfaces

SMS-GMSC:

-	Support invoking Nudm_EE service for "UE_MEMORY_AVAILABLE_FOR_SMS"

-	Support alert the SC according to the received notification of  "UE_MEMORY_AVAILABLE_FOR_SMS" event.

UDM:

-	Extend the EE service to support subscribe/unsubscribe/notify new event "UE_MEMORY_AVAILABLE_FOR_SMS".

-	Add new UE memory state for SMS update service operations in Nudm_UEContextManagement Service.

SMSF:

-	Support to invoke newly added service operations in Nudm_UEContextManagement.

## 6.11	Solution#11: Unsuccessful MO SM transfer

### 6.11.1	Introduction

This solution is to address Key Issue #2 "SBI-based SM MO message transfer". Introduce the solution for unsuccessful MO SM based on SBI in 5GC.

Figure 6.11.1-1 (from TS 3GPP 23.502 [4] clause 4.13.3.3) below is the baseline procedure of MO SM over NAS from TS 3GPP 23.502 [4]. We can observe that the interfaces between the SMSF and the SMS-IWMSC is still based on legacy protocol. This solution targets to provide the solution for Unsuccessful MO SM transfer based on the new SBI-based interfaces between the SMSF and the SMS-IWMSC/SC.

Figure 6.11.1-1: MO SM over NAS

### 6.11.2	Failure for MO SM transfer in SMS-IWMSC

SMS-IWMSC adds new Niwmsc_SMS service and UplinkSM service operation to support the transfer of the uplink SM based on SBI (see the description of the new service operation in solution b MO SM transfer of clause 6.5). SMS-IWMSC indicates the different errors for MO SM transfer in UplinkSM response according to the different failure scenarios which happened during MO SM transfer. The Application Errors used in Niwmsc_SMS UplinkSM response are defined in Table 6.11.2-1 below

Table 6.11.2-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| UNKNOWN_SERVICE_CENTRE_ADDRESS | 403 Forbidden | The delivery of the MO short message failed because SMS-SC was known. |
| SERVICE_CENTRE__CONGESTION | 403 Forbidden | The delivery of the MO short message failed because SMS-SC was in congestion. |
| USER_NOT_SERVICE_CENTER | 403 Forbidden | The delivery of the short message failed because the user didn't belongs to the SMS-SC. |
| FACILITY_NOT_SUPPORTED | 403 Forbidden | The delivery of the MO short message failed because of facility not supported. |
| UNREACHABLE_SMS_SC | 504 Gateway Timeout | The delivery of the MO short message failed because the response is timeout. |

If errors are indicated by the SMS-IWMSC, the SMSF shall send a failure report (i.e. a RP-ERROR message) to the UE, with the error cause coded as following map between errors indicated by SMS-IWMSC and error cause code in RP-ERROR message:

Table 6.11.2-2: MAP of Application errors

| Return error from SMS-IWMSC | Cause value in the RP-ERROR message |
| --- | --- |
| The Response from SMS-IWMSC is timeout Unspecified 4xx/5xx error code | 38 Network out of order |
| 403 Forbidden with FACILITY_NOT_SUPPORTED | 69 Requested facility not implemented |
| 403 Forbidden with UNKNOWN_SERVICE_CENTRE_ADDRESS | 1 Unassigned number |
| 403 Forbidden with SERVICE_CENTRE__CONGESTION | 42 Congestion |
| 403 Forbidden with USER_NOT_SERVICE_CENTER | 28 Unidentified subscriber |

NOTE:	The coding and the use of the RP-ERROR message is specified in 3GPP TS 24.011 [8].

### 6.11.3	Impacts on services, entities and interfaces

SMS-IWMSC：

-	Support new Niwmsc_SMS UplinkSM service operation and related Application errors.

SMSF:

-	Support invoking Niwmsc_SMS UplinkSM service operation and dealing with related Application errors and error codes.

# 7	Evaluations and Conclusions

This clause evaluates the potential solutions described in clause 5 and provides conclusions.

Each clause will evaluates the solutions for one key issue, and concludes on the solution for that key issue.

## 7.1	Evaluation and Conclusions of Solutions for Key Issue#1

Evaluation and Conclusions of Solutions for Key Issue#1

## 7.2	Evaluation and Conclusions of Solutions for Key Issue#2

Evaluation and Conclusions of Solutions for Key Issue#2

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-09 |  | C4-204587 |  |  |  | Implementation of C4-204117 and C4-204118 in CT4#99e | 0.1.0 |
| 2020-11 |  | C4-205772 |  |  |  | Implementation of C4-205277, C4-205283, C4-205284, C4-205659, C4-205720, C4-205721, C4-205722, C4-205723 and C4-205724 in CT4#101e | 0.2.0 |
| 2021-02 |  | C4-210244 |  |  |  | Implementation of C4-210207, C4-210208, C4-210202, C4-210094, C4-210229, C4-210231, C4-210085, C4-210089, C4-210104, C4-210195, C4-210210, C4-210224, C4-210235, C4-210238 and C4-210248 in CT4#101e-bis | 0.3.0 |