| 3GPP TR 29.866 V1.3.0 (2025-02) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Study on IMS Disaster Prevention and Restoration Enhancement (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

Introduction	7

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	9

3.1	Terms	9

3.2	Symbols	9

3.3	Abbreviations	9

4	Baseline	10

5	Key Issues	10

5.1	Key Issue #1: Continue service for registered users in case of HSS/UDM failure or overload	10

5.1.1	Description of the use case	10

5.1.2	Key issue definition	10

5.2	Key Issue #2: Handle all PCRF/PCF instances failure scenario with minimum impact to the registered IMS users	11

5.2.1	Description of the use case	11

5.2.2	Key issue definition	11

5.3	Key Issue #3: Speed up the restoration procedure of the terminating session when EPC/5GC NF fails	11

5.3.1	Description of the use case	11

5.3.2	Key issue definition	11

5.4	Key Issue #4: Preventing the IMS disaster in the HSS overload scenario	12

5.4.1	Description	12

5.4.2	Key issue definition	12

6	Solutions	12

6.0	Mapping of Solutions to Key Issues	12

6.1	Solution#1: Solution for optimizing Re-registration process in case of HSS/UDM failure or overload	13

6.1.1	Description	13

6.1.1.1	General	13

6.1.1.2	(Re)-registration procedure for routing without I-CSCF	13

6.1.1.3	(Re)-registration procedure for routing with I-CSCF	14

6.1.2	Impacts on services, entities and interfaces	15

6.1.2.1	Routing without I-CSCF	15

6.1.2.2	Routing with I-CSCF	15

6.1.3	Pros	15

6.1.4	Cons	15

6.2	Solution#2: Preventing the IMS disaster in the HSS overload scenario	15

6.2.1	Description	15

6.2.1.1	General	15

6.2.1.2	IMS initial registration of reducing HSS load	16

6.2.1.3	IMS third-party registration of reducing HSS load	17

6.2.2	Impacts on services, entities and interfaces	18

6.2.3	Pros	18

6.2.4	Cons	18

6.3	Solution#3: Solution for maintaining the IMS registration and call services uninterruptible in case of HSS/UDM failure	18

6.3.1	Description	18

6.3.1.1	General	18

6.3.1.2	Storage of S-CSCF Address	19

6.3.1.3	IMS initial registration procedure in case of HSS failure	19

6.3.1.4	IMS re-registration procedure in case of HSS failure	21

6.3.1.5	IMS MT procedure in case of HSS failure	21

6.3.2	Impacts on services, entities and interfaces	21

6.3.3	Pros	22

6.3.4	Cons	22

6.4	Solution #4: Solution for HSS/UDM bypass	22

6.4.1	Description	22

6.4.1.1	General	22

6.4.1.2	IMS deregistration procedure of HSS/UDM bypass	23

6.4.1.3	IMS initial registration procedure of HSS/UDM bypass	23

6.4.1.4	IMS re-registration procedure of HSS/UDM bypass	24

6.4.1.5	MT procedure of HSS/UDM bypass	24

6.4.2	Impacts on services, entities and interfaces	25

6.4.3	Pros	25

6.4.4	Cons	25

6.5	Solution #5: Solution for PCRF/PCF bypass	25

6.5.1	Description	25

6.5.1.1	General	25

6.5.1.2	Default IMS bearer establishment procedure of PCRF/PCF bypass	25

6.5.1.3	IMS MO procedure of PCRF/PCF bypass	26

6.5.1.4	IMS MT procedure of PCRF/PCF bypass	27

6.5.1.5	Dedicated IMS bearer termination procedure of PCRF/PCF bypass	27

6.5.2	Impacts on services, entities and interfaces	28

6.5.3	Pros	28

6.5.4	Cons	28

6.6	Solution #6: Solution for speeding up the restoration procedure of the terminating session when EPC/5GC NF fails	28

6.6.1	Description	28

6.6.1.1	General	28

6.6.1.2	Quick restoration procedure of IMS terminating session	29

6.6.2	Impacts on services, entities and interfaces	30

6.6.3	Pros	30

6.6.4	Cons	30

6.7	Solution#7: Enhance the P-CSCF restoration in the case of 5GC/EPC UP failure	31

6.7.1	Description	31

6.7.1.1	General	31

6.7.1.2	CN user plane failure detected	31

6.7.2	Impacts on services, entities and interfaces	32

6.7.3	Pros	32

6.7.4	Cons	32

6.8	Solution#8: Enhance the P-CSCF restoration in the case of 5GC/EPC failure	32

6.8.1	Description	32

6.8.1.1	General	32

6.8.1.2	PCRF/PCF detecting PGW-C/SMF failure	33

6.8.2	Impacts on services, entities and interfaces	34

6.8.3	Pros	34

6.8.4	Cons	34

6.9	Solution #9: Solution for speeding up the restoration procedure of the terminating session when EPC/5GC NF fails and EPC/5GC initiate the restoration procedures	35

6.9.1	Description	35

6.9.1.1	General	35

6.9.1.2	Quick restoration procedure for IMS Terminating session initiated by the 5GC/EPC	35

6.9.1.3	Reporting the outcome of the Packet Core Health-Check procedure to IMS Core NFs	37

6.9.2	Impacts on services, entities and interfaces	37

6.9.3	Pros	37

6.9.4	Cons	37

6.10	Solution #10: Dynamic extension of registration expiration timers by the Registration Management function	38

6.10.1	Description	38

6.10.1.1	General	38

6.10.1.2	Extension of registration expiration timers upon reception of congestion reports from HSS/UDM via Overload Control Information (OCI) HTTP header	38

6.10.1.3	Extension of registration expiration timers upon reception of congestion reports from HSS/UDM via HTTP status codes	41

6.10.1.4	Extension of expiration timers upon reception of congestion reports from HSS/UDM via Diameter Overload Indication Conveyance (DOIC) information	43

6.10.2	Impacts on services, entities and interfaces	46

6.10.3	Pros	46

6.10.4	Cons	46

7	Evaluations	47

7.1	Evaluation of Solutions for Key Issue#1	47

7.2	Evaluation of Solutions for Key Issue#2	48

7.3	Evaluation of Solutions for Key Issue#3	48

7.4	Evaluation of Solutions for Key Issue#4	49

8	Conclusions	49

8.1	Conclusion of Solutions for Key Issue#1	49

8.2	Conclusion of Solutions for Key Issue#2	50

8.3	Conclusion of Solutions for Key Issue#3	50

8.4	Conclusion of Solutions for Key Issue#4	50

Annex A (informative): Change history	51

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

This Technical Report aims to study potential enhancements on IMS Disaster Prevention and Restoration, to increase the stability of the IMS system.

# 1	Scope

The present document identifies IMS restoration mechanisms as specified in 3GPP TS 23.380 [2] and studies potential optimization solutions. The document will focus on the following aspects:

-	Create/enhance corresponding the restoration mechanism to support the normal use of voice, video and SMS services in the IMS network. Specifically, it adopts the context data of registered online users stored by HSS/UDM, S-CSCF, P-CSCF and MMTEL to bypass the involved faulty network functions or links. That supports the following procedures:

-	Re-registration procedure for registered online users;

-	Mobile terminated procedure for registered online users;

-	EPS fallback procedure for registered online users;

-	Initial registration procedure for registered online users after deregistration.

-	Create/enhance corresponding network disaster-prevention mechanisms, including:

-	UE specific registration analysis mechanism, to prevent registration with missing parameter or incorrect parameters repeatedly.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.380: "IMS Restoration Procedures".

[3]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[4]	3GPP TS 23.007: "Restoration procedures".

[5]	3GPP TS 29.228: "IP Multimedia (IM) Subsystem Cx and Dx interfaces; Signalling flows and message contents ".

[6]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[7]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[8]	IETF RFC 7683: "Diameter Overload Indication Conveyance".

[9]	3GPP TS 33.203: "Technical Specification Group Services and System Aspects; 3G security; Access security for IP-based services".

[10]	3GPP TS 23.203: "Policy and charging control architecture".

[11]	3GPP TS 29.214: "Policy and Charging Control over Rx reference point".

[12]	IETF RFC 6733: "Diameter Base Protocol".

[13]	IETF RFC 8582: "Diameter Overload Rate Control".

[14]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[15]	IETF RFC 777: "Internet Control Message Protocol".

[16]	3GPP TS 29.213: "Policy and Charging Control signalling flows and Quality of Service (QoS) parameter mapping".

[17]	3GPP TS 29.513: "5G System; Policy and Charging Control signalling flows and QoS parameter mapping; Stage 3".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

void

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

void

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core

5QI	5G QoS Identifier

AAA	Authorization-Authentication-Answer

AAR	Authorization-Authentication-Request

AMF	Access Management Function

AS	Application Server

AV	Authentication Vector

AVP	Attribute-Value Pair

CSCF	Call Server Control Function

EPC	Evolved Packet Core

HSS	Home Subscriber Server

I-CSCF	Interrogating-CSCF

iFC	initial Filter Criteria

IMS	IP Multimedia Subsystem

IMS-AGW	IMS Access Gateway

MAA	Multimedia-Auth-Answer

MAR	Multimedia-Auth-Request

MME	Mobility Management Entity

MMTEL	Multimedia Telephony

P-CSCF	Proxy-CSCF

P-GW	PDN Gateway

PCC	Policy and Charging Control

PCF	Policy Control Function

PCRF	Policy and Charging Rules Function

PFCP	Packet Forwarding Control Protocol

RAR	Re-Auth-Request

RR	RTCP Receiver Report

RTCP	Real-Time Control Protocol

QCI	QoS Class Identifier

S-CSCF	Serving CSCF

SAA	Server-Assignment-Answer

SAR	Server-Assignment-Request

SMF	Session Management Function

UDA	User Data Answer

UDM	Unified Data Management

UDR	User Data Request

UPF	User Plane Function

# 4	Baseline

3GPP TS 23.380 [2] has specified the procedures required in IMS to handle a S-CSCF or a P-CSCF service interruption scenario with minimum impact to the service to the end user. But IMS Restoration Procedures covering service interruption of other network elements (e.g. UDM, DRA, etc.) are not defined. So it is proposed to create/enhance corresponding restoration mechanism to network elements in the IMS network, to enhance the stability of IMS system.

Besides this, UE registration with missing parameter or incorrect parameters repeatedly may also lead to HSS/UDM overload, and thus make troubles to IMS. So it is proposed to create/enhance UE specific registration analysis for these abnormal behaviours and help to prevent IMS disasters.

# 5	Key Issues

## 5.1	Key Issue #1: Continue service for registered users in case of HSS/UDM failure or overload

### 5.1.1	Description of the use case

3GPP TS 23.380 [2] has already specified a couple of IMS restoration procedures for P-CSCF/S-CSCF failure scenario. However, the HSS/UDM failure or overload scenario has not been considered. In addition, P-CSCF/S-CSCF restoration procedures via HSS increases the HSS load which may further cause HSS overload.

HSS/UDM stores authentication data and subscription data of IMS subscribers. When one HSS/UDM instance fails, other HSS/UDM instances in the same set could provide services to I-CSCF/S-CSCF and IMS AS. In this case, there is no impact on IMS services. But once all the HSS/UDM instances in the set are unavailable, most IMS procedures (e.g. IMS (re-)registration, IMS deregistration, mobile terminating session, AS originating session, supplementary services such as IP SMS, etc.) are impacted which results the service failure. Similar situation happens to HSS/UDM overload scenario, since at that time the HSS/UDM is hard to handle the related procedures.

Even if HSS/UDM failure or overload, operators have strong requirement to continue IMS service for those users already registered to IMS, so that to minimize the service interruption due to HSS/UDM failure or overload. How to enhance existing procedure to support the operator requirement should be considered.

### 5.1.2	Key issue definition

This key issue will study the following aspects:

-	in the case of all HSS/UDM instances failure or overload, how to continue IMS service for registered users as much as possible;

-	which procedures can be enhanced to continue IMS service for registered users, and how to enhance the procedures.

## 5.2	Key Issue #2: Handle all PCRF/PCF instances failure scenario with minimum impact to the registered IMS users

### 5.2.1	Description of the use case

The reliability of PS domain is the prerequisite of the availability of IMS voice, video and SMS service. In 4G network, CSFB solution could be used to provide voice call service for subscribers when EPC network failure causes that VoLTE is unavailable. But CSFB solution could not be used anymore after 2/3G sunset. Therefore, it’s important to study how to minimize the impact to IMS services when some network function in EPC/5GC network fails, especially when all network functions of this kind are faulty.

PCRF/PCF provides QoS guarantee to IMS services, which are defined in 3GPP TS 23.503 [7]. When one PCRF/PCF instance fails, the other PCRF/PCF instances in the set could provide services to P-CSCF and there is no impact on IMS services. But once all the PCRF/PCF instances in the set are unavailable, the dedicated bearers of voice/video media could not be established which causes that all the newly initiated calls fail. This key issue will study the solution addressing this case.

### 5.2.2	Key issue definition

This key issue will study the following aspects:

-	in the case of all PCRF/PCF instances failure or overload, how to continue IMS service for registered users as much as possible;

-	which procedures can be enhanced to continue IMS service for registered users, and how to enhance the procedures.

## 5.3	Key Issue #3: Speed up the restoration procedure of the terminating session when EPC/5GC NF fails

### 5.3.1	Description of the use case

In most of EPC/5GC NF failure scenarios, the UE could invoke re-attach and re-registration procedures quickly if

-	the UE initiates a new IMS session and some SIP error code is sent by the IMS network; or

-	the EPC/5GC network detects the failure and kicks off the related UEs to re-attach.

Then the UE’s service will be restored soon.

But in some cases, the re-attach and re-registration procedures will not be invoked immediately. In that case, the UE’s terminating session will be unavailable until its IMS registration is expired and it invokes the re-registration procedure. If the UE’s registration expiration time is 6000 seconds, the unavailable time is 50 minutes in the worst case. It’s important to find a solution to speed up the restoration procedure of the terminating session when EPC/5GC NF fails.

### 5.3.2	Key issue definition

This key issue will study the following aspects:

-	Specify the mechanism that speeds up the restoration procedure of the terminating session when EPC/5GC NF fails.

NOTE:	The mechanism to speed up the restoration procedure should not impact the UE and introduce overload of EPC/5GC.

## 5.4	Key Issue #4: Preventing the IMS disaster in the HSS overload scenario

### 5.4.1	Description

3GPP TS 23.380 [2] has already specified a couple of IMS restoration procedures for the P-CSCF/S-CSCF failure scenario. However, how to prevent the IMS disaster has not been considered in the HSS overload scenario.

In addition to overload control mechanisms in IETF RFC 7683 [8], the following flow-reduction mechanisms for overload control are defined. If many UEs initiate the initial REGISTER request to the IMS network, a large number of signalling interactions with HSS will accelerate the occurrence of signalling storm. Thus, it is necessary to consider the case in which the signalling interactions between NFs and HSS can be simplified to reduce the HSS load. For example, if the IP address assigned to the UE by the EPC/5GC changes when the UE undergoes a location move, the UE will initiate an initial REGISTER request to the IMS network. At the same time, the used S-CSCF is the same as the last used S-CSCF. Therefore, the stored user data in the S-CSCF can be reused to complete the initial registration procedure, further reducing unnecessary signalling interaction with HSS.

### 5.4.2	Key issue definition

This key issue will study the following aspects:

-	How to reduce the load of HSS to prevent IMS disaster when the HSS overloads. For example, by reducing signal flow via Cx and Sh interfaces.

NOTE:	This solution of the KI is only for the HSS overload scenario, which does not impact the existing procedures for normal cases.

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 |
| 1 | X |  |  |  |
| 2 |  |  |  | X |
| 3 | X |  |  |  |
| 4 | X |  |  |  |
| 5 |  | X |  |  |
| 6 |  |  | X |  |
| 7 |  |  | X |  |
| 8 |  |  | X |  |
| 9 |  |  | X |  |
| 10 |  |  |  | X |

## 6.1	Solution#1: Solution for optimizing Re-registration process in case of HSS/UDM failure or overload

### 6.1.1	Description

#### 6.1.1.1	General

The solution is to address the Key Issue #1: Continue service for registered users in case of HSS/UDM failure or overload.

3GPP TS 23.380 [2] has already specified a couple of IMS restoration procedures for P-CSCF/S-CSCF failure scenario. However, the HSS/UDM failure or overload scenario has not been considered. In addition, the existing P-CSCF/S-CSCF restoration procedures via HSS/UDM increases the HSS/UDM load which may further cause HSS/UDM overload.

In the case of HSS/UDM failure, most IMS procedures (e.g. IMS (re-)registration, IMS deregistration, mobile terminating session, AS originating session, supplementary services such as IP SMS, etc.) are impacted which results in the service failure.

This solution considers to continue IMS service for registered UE, in the following procedures:

-	(re)-registration procedure for routing without I-CSCF;

-	(re)-registration procedure for routing with I-CSCF;

Selection of routing without I-CSCF or routing with I-CSCF in the scenario of HSS/UDM failure or overload is based on operation policy.

#### 6.1.1.2	(Re)-registration procedure for routing without I-CSCF

The below Figure 6.1.1.2-1 illustrates the re-registration procedure for routing without I-CSCF when HSS/UDM failure or overload is detected.

Figure 6.1.1.2-1: Re-registration procedure for routing without I-CSCF

1.	Prior to the expiration of a user's registration, the user sends a re-registration message.

2.	The P-CSCF assesses the user's registration status. If the user is already registered, it forwards the re-registration message directly to the S-CSCF.

3-4. Authentication and registration timer update process are consistent with procedures outlined in Annex A of 3GPP TS 29.228 [5] and more details are described in 3GPP TS 24.229 [6] and 3GPP TS 23.228 [3].

5-6. According to the strategy in 3GPP TS 23.228 [3], step 5-6 are optional as an optimization. If the S-CSCF chooses to interact with HSS/UDM, when the HSS/UDM has no response or returns an appropriate error code, the S-CSCF shall skip the interactions with HSS/UDM and return 200 OK to the UE.

7-8. The S-CSCF returns 200 OK to the UE.

#### 6.1.1.3	(Re)-registration procedure for routing with I-CSCF

The below Figure 6.1.1.3-1 illustrates the Re-registration procedure for routing with I-CSCF when HSS/UDM failure or overload is detected.

Figure 6.1.1.3-1: Re-registration procedure for routing with I-CSCF

1.	Prior to the expiration of a user's registration, the user sends a re-registration message.

2.	The P-CSCF assesses the user's registration status. If the user is already registered, the P-CSCF adds the address of S-CSCF to a new header field and forwards the re-registration message to the I-CSCF.

3.	The I-CSCF identifies the new header field within the message. If it presents, I-CSCF forwards the re-registration message to the S-CSCF address obtained from the new header field.

4-5.	Authentication and registration timer update process are consistent with procedures outlined in Annex A of 3GPP TS 29.228 [5] and more details are described in 3GPP TS 24.229 [6] and 3GPP TS 23.228 [3].

6-7.	According to the strategy in 3GPP TS 23.228 [3], step 6-7 are optional as an optimization. If the S-CSCF chooses to interact with HSS/UDM, when the HSS/UDM has no response or returns an appropriate error code, the S-CSCF shall skip the interactions with HSS and return 200 OK to the UE.

8-10.	The S-CSCF returns 200 OK to the UE.

### 6.1.2	Impacts on services, entities and interfaces

If HSS/UDM failure or overload is detected, some enhancements are needed for the optimization.

#### 6.1.2.1	Routing without I-CSCF

The requirements for the modification of P-CSCF:

-	Add functionality for directly routing re-registration messages to S-CSCF.

The requirements for the modification of S-CSCF:

-	Bypass the HSS/UDM interaction during the re-registration procedure.

#### 6.1.2.2	Routing with I-CSCF

The requirements for the modification of P-CSCF:

-	Including the address of S-CSCF into a new header field.

The requirements for the modification of I-CSCF:

-	Identifying the new header field, if included, forwarding the re-registration message to the S-CSCF address obtained from the new header field;

The requirements for the modification of S-CSCF:

-	Bypass the HSS/UDM interaction during the re-registration procedure.

### 6.1.3	Pros

This solution provides different efficient ways to bypass the HSS/UDM if HSS/UDM failure or overload is detected.

### 6.1.4	Cons

The cons of the solution:

-	Enhancements to P-CSCF, I-CSCF and S-CSCF are required.

-	The method of routing without I-CSCF deviates from the signaling flows specified in 3GPP TS 23.228 [3], so this approach needs to be discussed in Stage 2.

## 6.2	Solution#2: Preventing the IMS disaster in the HSS overload scenario

### 6.2.1	Description

#### 6.2.1.1	General

This solution is proposed to address Key Issue #2 "Preventing the IMS disaster in the HSS overload scenario".

If the response failure rate from HSS arrives at the thresholds based on the indicators analysis by internal or external mechanisms, or the HSS identifies that it is in an overload situation by implementing specific means, such as the level of usage of internal resources (CPU, memory), the access to external resources, etc., as described in 3GPP TS 29.228 [5] Annex H and IETF RFC 7683 [8], or other internal mechanisms of S-CSCF, the S-CSCF will reduce the requests traffic to the HSS. The HSS as a reporting node determines the specific contents of OC-OLR AVP in overload reports and sends OC-OLR AVPs to I/S-CSCF as reacting nodes. The specific thresholds of HSS overload are defined based on the operator's policy.

This solution considers reducing the Diameter signalling interactions between S-CSCF/AS and HSS using stored available AVs and user profile, which contains two procedures as follows and discusses how to decrease HSS load in these procedures:

-	IMS initial registration procedure.

-	IMS third-party registration procedure.

#### 6.2.1.2	IMS initial registration of reducing HSS load

Figure 6.2.1.2-1 below illustrates the IMS initial registration procedure in the case of the presence of user data in the S-CSCF.

Figure 6.2.1.2-1: Initial registration procedure under the presence of user data in the S-CSCF

1-5.	The UE initiates an initial REGISTER request to the P-CSCF, the procedures between P-CSCF, I-CSCF, S-CSCF, and HSS, referring to procedures in 3GPP TS 29.228 [5] Annex A and more details as described in 3GPP TS 24.229 [6] and 3GPP TS 23.228 [3].

6.	The S-CSCF receives a REGISTER request with the tag of "integrity-protected=no", which means that the registration type is initial REGISTER described in 3GPP TS 33.203 [9]. When the S-CSCF detects the HSS overload, the S-CSCF shall initiate the initial registration optimized mechanism to check whether any available AV and user profiles are stored.

7.	If there are stored available AVs, the S-CSCF shall skip steps 7-8. If there are no stored available AVs, the S-CSCF shall send MAR request to obtain AVs to HSS.

8.	The HSS shall generate AVs, including RAND, XRES, CK, IK, and AUTN, then send them to the S-CSCF.

9.	The S-CSCF selects one group of AVs, and sends the 401 (Unauthorized) response to the initial REGISTER request to the I-CSCF.

10-16.	These steps as described in 3GPP TS 24.229 [6].

17.	The S-CSCF sends the SAR message to the HSS.

18.	The HSS shall send the SAA response to the S-CSCF.

-	If the S-CSCF receives the Result-Code in the SAA response as DIAMETER_UNABLE_TO_DELIVER (3002) or DIAMETER_UNABLE_TO_COMPLY (5012), or time-out of SAA response, the S-CSCF shall ignore the error response and send 200 OK response to the I-CSCF. Meanwhile, the S-CSCF shall set the UPDATE RESTORATION INFO flag as PENDING while next receives a REGISTER request, the S-CSCF resends the SAR request to the HSS.

19.	The S-CSCF shall return the 200 OK response to the I-CSCF.

20-21.	The I-CSCF sends 200 OK response to the P-CSCF and the P-CSCF sends 200 OK response to the UE.

#### 6.2.1.3	IMS third-party registration of reducing HSS load

Upon successful completion of the initial registration procedure, the S-CSCF shall send a third-party REGISTER request to the MMTel AS and IP-SM-GW according to the stored context of the UE. The diameter signalling interaction between MMTel AS/ IP-SM-GW and HSS can be further reduced in case of HSS overload. Figure 6.2.1.3-1 below illustrates the third-party registration procedure under the presence of user data in MMTel AS/IP-SM-GW.

Figure 6.2.1.3-1: Third-party registration procedure under the presence of user data in MMTel AS/IP-SM-GW

1.	S-CSCF sends a third-party REGISTER request to the MMTel AS. UDR and SNR requests needn't be sent to obtain user data due to the presence of user data in the MMTel AS.

2.	MMTel sends the 200 OK response to the S-CSCF.

3.	S-CSCF sends a third-party REGISTER request to the IP-SM-GW. UDR and SNR requests also needn't be sent to obtain UE's data due to the presence of user data in the IP-SM-GW.

4.	IP-SM-GW sends the 200 OK response to the S-CSCF.

### 6.2.2	Impacts on services, entities and interfaces

Compared to procedures defined in 3GPP TS 23.228 [3], some enhancements of the S-CSCF are needed for the optimization.

1.	Determining whether stored available AVs present while the S-CSCF receives an initial REGISTER request.

2.	Optimizing the handling mechanism of SAR/SAA messages while the S-CSCF receives an initial REGISTER request during the registration procedure.

3.	If the S-CSCF receives the Result-Code in the SAA response as DIAMETER_UNABLE_TO_DELIVER (3002) or DIAMETER_UNABLE_TO_COMPLY (5012), or time-out of SAA response, the S-CSCF shall ignore the error response and send 200 OK response to the I-CSCF. Meanwhile, the S-CSCF shall set the UPDATE RESTORATION INFO flag as PENDING while next receives a REGISTER request, the S-CSCF resends the SAR request to the HSS.

4.	The third-party REGISTER request is sent to the same AS from the S-CSCF as the process of normal use by the S-CSCF the last used AS according to UE's iFCs, which can further simplify the Diameter signalling (Sh) between AS and HSS.

### 6.2.3	Pros

The pros of the solution:

-	The solution reduces MAR/MAA signalling over Cx interface, and UDR/UDA and SNR/SNA signalling over Sh interface the UE-initiated initial registration procedure in case of HSS overload with stored user data. Especially for Sh interface signalling, which significantly reduces the signalling amplification factor of the Diameter signalling network.

-	The solution can further decrease the probability of signalling storm and accelerate failure restoration for the IMS network.

### 6.2.4	Cons

The cons of the solution:

-	Enhancements to S-CSCF are required.

## 6.3	Solution#3: Solution for maintaining the IMS registration and call services uninterruptible in case of HSS/UDM failure

### 6.3.1	Description

#### 6.3.1.1	General

This solution is proposed to address Key Issue #1 "Continue service for registered users in case of HSS/UDM failure or overload".

This solution considers to continue IMS service for registered UE, in the following procedures:

-	IMS initial registration procedure;

-	IMS re-registration procedure;

-	IMS MT procedure.

During the IMS (re-)registration and MT services procedures, I-CSCF will retrieve the S-CSCF address in the terminating network from the HSS of the terminating UE. To ensure successful S-CSCF address retrieval in case of HSS/UDM failure, the S-CSCF address is stored in a new AS during the registration process.

#### 6.3.1.2	Storage of S-CSCF Address

The below Figure 6.3.1.2-1 illustrates the storage of S-CSCF address during the IMS registration procedure.

Figure 6.3.1.2-1: Storage of S-CSCF address in initial registration procedure

1-2.	UE has registered to the IMS network.

3-4.	S-CSCF sends the third-party registration request to a new AS based on the iFC information. Upon a successful registration, the AS will store the S-CSCF address, IMSI, MSISDN for the UE.

NOTE:	The new AS is unique and its address is pre-configured in the I-CSCF, dedicated for HSS bypass in case of HSS failure.

#### 6.3.1.3	IMS initial registration procedure in case of HSS failure

Although the UE has registered to the IMS network, it still has the possibility to initiate an initial registration request in some cases. For example, the exchange of P-CSCF, PDN connection re-established, etc. At the same time, the used S-CSCF is the same as the last used S-CSCF and the new AS has restored the UE registration-related data (e.g. S-CSCF address, IMSI, MSISDN, etc.). Therefore, the UE data on the AS can be fully used for the completion of the initial registration procedure for the UE. The below Figure 6.3.1.3-1 illustrates the initial registration procedure of HSS bypass.

NOTE:	If a UE does not have an allocated S-CSCF, this solution for IMS initial registration does not work.

Figure 6.3.1.3-1: IMS initial registration procedure of HSS bypass

1-2.	The UE initiates an initial REGISTER request to the P-CSCF and the P-CSCF forwards the initial REGISTER request to the I-CSCF.

3-4.	I-CSCF fails to send a UAR or doesn’t receive a corresponding response from HSS, then it forwards the REGISTER request to the AS based on the pre-configured address of the AS.

5a.	AS supports forwarding the REGISTER request to the S-CSCF according to the UE registration-related data (e.g. S-CSCF address, IMSI, MSISDN, etc.). The UE data is obtained directly or indirectly from the ISC interface (S-CSCF-AS).

5b-5c.	AS also supports returning the 305 message with the S-CSCF address to the I-CSCF. Then the I-CSCF sends the REGISTER to the S-CSCF.

6a-6b.	During the IMS initial registration procedure, the HSS will send an ordered array of authentication vectors to the S-CSCF as described in clause 6.1.1 of 3GPP TS 33.203 [9]. Therefore, when the used S-CSCF is the same as the last used S-CSCF and existing available AVs, the S-CSCF selects one group of available AVs and returns the 401 (Unauthorized) response to the initial REGISTER request to the AS. Then the AS forwards the 401 (Unauthorized) response to the I-CSCF.

6c.	The S-CSCF selects one group of available AVs and returns the 401 (Unauthorized) response to the initial REGISTER request to the I-CSCF.

7-10.	These steps are the same as the procedure of solution 2 described in clause 6.2.1.2.

11-13.	These steps are the same as the 3 to 6 steps above.

14-16.	The S-CSCF shall return the 200 OK response to the UE by the AS/I-CSCF and the P-CSCF.

#### 6.3.1.4	IMS re-registration procedure in case of HSS failure

According to 3GPP TS 24.229 [6], upon the initial registration being successful, the P-CSCF will store the address of the S-CSCF serving the specific subscriber. The re-registration procedures are then the same as in solution 1. Either the procedure with I-CSCF described in clause 6.1.1.2 or the procedure without I-CSCF described in clause 6.1.1.3 may be used.

#### 6.3.1.5	IMS MT procedure in case of HSS failure

The below Figure 6.3.1.5-1 illustrates the MT procedure of HSS bypass.

Figure 6.3.1.5-1: IMS MT procedure of HSS bypass

1.	I-CSCF in the terminating network receives an incoming INVITE request.

2-3.	I-CSCF fails to send an LIR or doesn’t receive a corresponding response from HSS, then it forwards the INVITE request to the AS based on the pre-configured address of the AS.

4a.	AS supports forwarding the INVITE request to the S-CSCF according to the UE registration-related data (e.g. S-CSCF address, IMSI, MSISDN, etc.). The UE data is obtained directly or indirectly from the ISC interface (S-CSCF-AS).

4b-4c.	AS also supports returning the 305 message with the S-CSCF address to the I-CSCF. Then the I-CSCF sends the INVITE to the S-CSCF.

5-6.	S-CSCF forwards the INVITE request to UE B through the P-CSCF.

7.	18X/200 OK/ACK procedure proceeds.

### 6.3.2	Impacts on services, entities and interfaces

The requirement for I-CSCF:

-	The capability to forward the REGISTER and INVITE request to the AS in order to obtain the S-CSCF address, IMSI, and MSISDN for serving the subscriber.

The requirement for the AS:

-	The capability to forward the REGISTER and INVITE request to the S-CSCF serving the subscriber based on the UE data obtained directly or indirectly from the ISC interface.

or

-	The capability to return the 305 message with the S-CSCF address to the I-CSCF based on the UE data obtained directly or indirectly from the ISC interface.

### 6.3.3	Pros

The pros for both Option A and Option B:

-	This solution provides an efficient way to bypass the failed HSS and associated paths.

-	This solution can bypass the HSS to keep the IMS (re-)registration and MT procedures uninterruptible when all the HSS instances fail. It supplements and improves solution 1.

The pros only for Option A:

-	The AS can directly route REGISTER and INVITE messages to the S-CSCF of the UE.

The pros only for Option B:

-	The AS first needs to return 305 (Use Proxy) with the S-CSCF address, and then the I-CSCF routes REGISTER and INVITE messages to the S-CSCF.

### 6.3.4	Cons

The cons for both Option A and Option B:

-	Enhancements to S-CSCF to configure the iFC of the new AS in the third-party registration, which will increase the overhead of registration signalling.

-	Enhancements to I-CSCF to forward REGISTER and INVITE requests to the AS.

-	The address of the AS needs to be preconfigured for I-CSCF in this solution.

-	Restoration of the new AS is needed to be considered.

The cons only for Option A:

-	Enhancements to the AS to forward REGISTER and INVITE requests to the S-CSCF serving the subscriber.

The cons only for Option B:

-	Enhancements to the AS to return 305 (Use Proxy) message to the I-CSCF.

## 6.4	Solution #4: Solution for HSS/UDM bypass

### 6.4.1	Description

#### 6.4.1.1	General

This solution addresses Key Issue #1 "Continue service for registered users in case of HSS/UDM failure or overload".

Solution 1 has provided a solution addressing Key Issue #1 too. But it only considers the case that the P-CSCF has the address of the S-CSCF serving the subscriber when it receives a (re-)REGISTER request. This solution considers the case that the P-CSCF does not have the address of the S-CSCF address serving the subscriber. The typical scenario is that a UE attaches to a new P-GW for roaming or mobility.

This solution contains four procedures as follows and discusses how to bypass HSS/UDM in these procedures:

-	IMS deregistration procedure;

-	IMS initial registration procedure;

-	IMS re-registration procedure;

-	IMS terminating session procedure.

The MO procedure need not be considered for it has no interaction with the HSS/UDM. That is, there is no impact to the originating session procedure of VoLTE/VoNR when all the HSS/UDM instances fail.

#### 6.4.1.2	IMS deregistration procedure of HSS/UDM bypass

According to the IMS de-registration procedure specified in clause 5.3.1 of 3GPP TS 23.228 [3], the S-CSCF shall remove all subscription information related to this specific Public User Identity.

To address the abnormal cases such as HSS/UDM failure or overload discussed in Key Issue #1, it’s recommended that the S-CSCF delays removing all subscription information related to this specific Public User Identity in the de-registration procedure. This is a prerequisite for subsequent processes to be implemented. The delay time depends on the operator's policy.

#### 6.4.1.3	IMS initial registration procedure of HSS/UDM bypass

Figure 6.4.1.3-1 below illustrates the IMS initial registration procedure in the case of all the HSS/UDM instances failure.

Figure 6.4.1.3-1: IMS initial registration procedure of HSS/UDM bypass

1-3.	UE A initiates an initial REGISTER request to the P-CSCF and the P-CSCF sends the initial REGISTER request to the I-CSCF. The I-CSCF detects that all the HSS/UDM instances in the network fail.

4-5.	Then the I-CSCF sends OPTIONS requests to all the S-CSCFs in the network to query which has the user data of UE A. The OPTIONS requests contain the identity of UE A.

6-7.	The S-CSCF-2 which has the subscription information of UE A will returns a 200 OK response to the I-CSCF, including the address of itself in Contact header field, other S-CSCFs will return a 404 Not Found response to the I-CSCF.

NOTE:	The I-CSCF can use some mechanism to improve the query efficiency of OPTIONS. For example, it can use the same S-CSCF selection algorithm used after it gets the S-CSCF capability set from the HSS/UDM, select the first S-CSCF it queries, or it sends out the OPTOINS requests in batches, that is, it sends the next batch of OPTIONS requests out only when all it receives are 404 Not Found responses and the S-CSCFs in the same pool with it are queried in the first batch if the pool interworking is used.

8.	Then the I-CSCF routes the initial REGISTER request to the S-CSCF-2's address carried in the received 200 OK response.

9.	The S-CSCF-2 returns a 401 Unauthorized response to the I-CSCF, including the address of itself in Service-Route header field.

10-11.	The 401 Unauthorized response will be returned to UE A via the P-CSCF. Then the P-CSCF knows the address of the S-CSCF-2 serving UE A.

Then UE A sends an initial REGISTER request to the P-CSCF and the procedure is same with the procedure of solution 1. Both the procedure with I-CSCF described in clause 6.1.1.2 and the procedure without I-CSCF described in clause 6.1.1.3 can be used here.

#### 6.4.1.4	IMS re-registration procedure of HSS/UDM bypass

According to 3GPP TS 24.229 [6], the P-CSCF will store the address of the S-CSCF serving the specific subscriber after the initial registration is successful. Then the P-CSCF can route the re-REGISTER request to the corresponding S-CSCF directly or it can carry the address of the corresponding S-CSCF in some header field of the re-REGISTER request and forward the re-REGISTER request to the I-CSCF, then the I-CSCF can routes the re-REGISTER request to the corresponding S-CSCF based on the address added by the P-CSCF.

#### 6.4.1.5	MT procedure of HSS/UDM bypass

Figure 6.4.1.5-1 below illustrates the MT procedure in the case of all the HSS/UDM instances failure.

Figure 6.4.1.5-1: IMS MT procedure of HSS/UDM bypass

1-2.	The I-CSCF in the terminating network receives an incoming INVITE request and it detects that that all the HSS/UDM instances fail.

3-4.	Then the I-CSCF sends OPTIONS requests to all the S-CSCFs in the network to query which has the user data of UE B. The OPTIONS requests contain the identity of UE B.

5-6.	The S-CSCF-2 which has the subscription information of UE B will returns a 200 OK response to the I-CSCF, including the address of itself in Contact header field, other S-CSCFs will return a 404 Not Found response to the I-CSCF.

NOTE:	The I-CSCF can use some mechanism to improve the query efficiency of OPTIONS. For example, it can use the same S-CSCF selection algorithm used after it gets the S-CSCF capability set from the HSS/UDM, select the first S-CSCF it queries, or it sends out the OPTOINS requests in batches, that is, it sends the next batch of OPTIONS requests out only when all it receives are 404 Not Found responses, and the S-CSCFs in the same pool with it are queried in the first batch if the pool interworking is used.

7.	Then the I-CSCF routes the incoming INVITE request to the S-CSCF-2's address carried in the 200 OK response it received.

8-9.	The S-CSCF sends the INVITE request to UE B via the P-CSCF.

10.	18X/200 OK/ACK procedure proceeds.

### 6.4.2	Impacts on services, entities and interfaces

The requirements for I-CSCF:

-	Query the subscriber's serving S-CSCF from S-CSCF through OPTIONS/200 OK interaction when detecting the HSS/UDM failure.

The requirements for S-CSCF:

-	Delay removing all subscription information related to this specific Public User Identity in the de-registration procedure.

-	Return the address of itself in the 200 OK response of OPTIONS request from I-CSCF if it has the subscription information of the specific subscriber.

### 6.4.3	Pros

This solution can bypass the HSS/UDM to continue the IMS (re-)registration and the IMS terminating session procedure when all the HSS/UDM instances fail, even though P-CSCF does not have the address of the S-CSCF serving the subscriber. It supplements and improves solution 1.

### 6.4.4	Cons

Enhancements to I-CSCF and S-CSCF are required.

Additional signaling overhead is introduced.

## 6.5	Solution #5: Solution for PCRF/PCF bypass

### 6.5.1	Description

#### 6.5.1.1	General

This solution addresses Key Issue #2 "Handle all PCRF/PCF instances failure scenario with minimum impact to the registered IMS users". It covers the following procedures which need to interact with PCRF/PCF and discusses how to bypass PCRF/PCF in these procedures:

-	Default IMS bearer establishment procedure for IMS signaling;

-	IMS originating session procedure in which the dedicated IMS bearer(s) need to be established for IMS media of the originating UE;

-	IMS terminating session procedure in which the dedicated IMS bearer(s) need to be established for IMS media of the terminating UE.

-	IMS session release procedure in which the dedicated IMS bearer(s) need to be terminated for both the originating UE and the terminating UE.

#### 6.5.1.2	Default IMS bearer establishment procedure of PCRF/PCF bypass

Before an IMS subscriber initiates a registration on an IMS network, a default IMS bearer must be established. Figure 6.5.1.2-1 below illustrates the default IMS bearer establishment procedure in the case of all the PCRF/PCF instances failure.

Figure 6.5.1.2-1: Default IMS bearer establishment procedure of PCRF/PCF bypass

1-2.	UE A sends a PDN Connectivity Request message to the MME/AMF to request a PDN connection. The MME/AMF allocates a bearer ID and sends a Create Session Request message to the PGW-C/SMF.

3.	The PGW-C/SMF detects that all the PCRF/PCF instances in the network fail, then it uses local PCC policy to establish a default IMS bearer (QCI/5QI=5) for IMS signaling.

4.	The subsequent procedure of the default IMS bearer establishment proceeds.

5.	After the default IMS bearer is established, UE A initiates the initial registration procedure which specified in 3GPP TS 23.228 [3].

#### 6.5.1.3	IMS MO procedure of PCRF/PCF bypass

Figure 6.5.1.3-1 below illustrates the IMS MO procedure in the case of all the PCRF/PCF instances failure.

Figure 6.5.1.3-1: IMS MO procedure of PCRF/PCF bypass

1-3.	UE A initiates an initial INVITE request to the P-CSCF to establish an IMS session. The P-CSCF routes the INVITE request to the I-/S-CSCF. Then the I-/S-CSCF returns a 183 response to the P-CSCF.

4-7.	The P-CSCF detects that all the PCRF/PCF instances fail, and starts the PCRF/PCF bypass procedure by sending an enhanced H.248 MOD request to the IMS-AGW. After receiving the enhanced H.248 MOD request, the IMS-AGW sends RTCP RR (Receiver Report) packets to UE A and returns an H.248 MOD response to the P-CSCF.

8-9.	The PGW-U/UPF detects the RTCP RR packets and finds there is no dedicated IMS bearer which has been established, then it starts the PCRF/PCF bypass procedure by sending a PFCP Session Report Request to the PGW-C/SMF.

10-12.	The PGW-C/SMF uses local PCC policy to establish the dedicated IMS bearer for IMS media and returns a PFCP Session Report Response to the PGW-U/UPF. The subsequent procedure of the dedicated IMS bearer establishment proceeds.

13-14.	The P-CSCF sends the 183 response to UE A and the subsequent procedure of IMS originating session proceeds.

#### 6.5.1.4	IMS MT procedure of PCRF/PCF bypass

Figure 6.5.1.4-1 below illustrates the IMS terminating session procedure in the case of all the PCRF/PCF instances failure.

Figure 6.5.1.4-1: IMS MT procedure of PCRF/PCF bypass

1-4.	The I-/S-CSCF in the terminating network receives an incoming INVITE request and routes it to UE B through the P-CSCF. UE B sends a 183 response to the P-CSCF.

5-8.	The P-CSCF detects that all the PCRF/PCF instances fail, and starts the PCRF/PCF bypass procedure by sending an enhanced H.248 MOD request to the IMS-AGW. After receiving the enhanced H.248 MOD request, the IMS-AGW sends RTCP RR (Receiver Report) packets to UE B and returns an H.248 MOD response to the P-CSCF.

9-10.	The PGW-U/UPF detects the RTCP RR packets and finds there is no dedicated IMS bearer which has been established, then it starts the PCRF/PCF bypass procedure by sending a PFCP Session Report Request to the PGW-C/SMF.

11-12.	The PGW-C/SMF uses local PCC policy to establish the dedicated IMS bearer for IMS media and returns a PFCP Session Report Response to the PGW-U/UPF. The subsequent procedure of the dedicated IMS bearer establishment proceeds.

13-15.	The P-CSCF sends the 183 response to the originating network through the I-/S-CSCF and the subsequent procedure of IMS terminating session proceeds.

#### 6.5.1.5	Dedicated IMS bearer termination procedure of PCRF/PCF bypass

When releasing the established IMS session, the normal dedicated IMS bearer termination procedure cannot be performed in the case of all PCRF/PCF instances failure. The PGW-U/UPF need to detect the traffic over the dedicated IMS bearers. When it finds that there is no traffic over some dedicated IMS bearer for longer than the timer which is configured by the operator, the PGW-C/SMF initiate bearer termination procedure for the dedicated IMS bearer.

### 6.5.2	Impacts on services, entities and interfaces

The requirements for P-CSCF:

-	Support to detect that all the PCRF/PCF instances fail and start the PCRF/PCF bypass procedure by sending an enhanced H.248 MOD request to the IMS-AGW.

The requirements for IMS-AGW:

-	Support RTCP Receiver Report packets to UE when receiving an enhanced H.248 MOD request.

The requirements for PGW-U/UPF:

-	Support to detect RTCP RR packets and start the PCRF/PCF bypass procedure by sending a Session Report Request to the PGW-C/SMF when there is no dedicated IMS bearer which has been established.

-	Support to detect the traffic over the dedicated IMS bearers and initiate bearer termination procedure for the dedicated IMS bearer when it finds that there is no traffic over some dedicated IMS bearer for longer than the timer.

The requirements for PGW-C/SMF:

-	Support to bypass PCRF/PCF by using local PCC policy to establish the default IMS bearer for IMS signaling when detecting that all the HSS/UDM instances fail.

-	Support to bypass PCRF/PCF by using local PCC policy to establish the dedicated IMS bearer for IMS media when receiving a Session Report Request from the PGW-U/UPF.

### 6.5.3	Pros

This solution can bypass the PCRF/PCF to continue the default IMS bearer establishment, the IMS originating session and the IMS terminating session procedure when all the PCRF/PCF instances fail.

### 6.5.4	Cons

-	This solution introduces the extra requirements described in clause 6.5.2 to the PGW-U/UPF, PGW-C/SMF, P-CSCF and IMS-AGW when all the PCRF/PCF instances fail and it will increase the system overhead of these NFs.

-	This solution does not support scenarios where there is a need to interact with the PCRF/PCF, e.g., for Network Provided Location Information as specified in 3GPP TS 23.203 [10] and 3GPP TS 23.503 [7], or to get the roaming status of the UE, or to modify the QoS parameters of the media session, or build the new dedicated bearer(s) after the media re-negotiation.

## 6.6	Solution #6: Solution for speeding up the restoration procedure of the terminating session when EPC/5GC NF fails

### 6.6.1	Description

#### 6.6.1.1	General

This solution addresses Key Issue #3 "Speed up the restoration procedure of the terminating session when EPC/5GC NF fails".

In most of EPC/5GC NF failure scenarios, the UE could invoke re-attach and re-registration procedures quickly after the failure is restored if

-	the UE initiates a new IMS session and some SIP error code is returned by the IMS network; or

-	the EPC/5GC network detects the failure and kicks off the related UEs to re-attach.

Then the UE’s service will be restored soon. In addition, the UE’s terminating session can be available if the operator’s network supports CS Fallback.

But when the operator’s network does not support CS Fallback, the re-attach and re-registration procedures will not be invoked immediately, the UE's terminating session will be unavailable until its IMS registration is expired and it invokes the re-registration procedure. If the UE’s registration expiration time is 6000 seconds, the unavailable time is 50 minutes in the worst case, as specified in clause 5.1.1.4.1 of 3GPP TS 24.229 [6].

The solution can avoid this situation which makes use of the HSS based P-CSCF Restoration procedure in EPC network specified in clause 5.4.2.1 of 3GPP TS 23.380 [2] or the UDM/HSS based P-CSCF Restoration procedure in 5GC network specified in clause 5.8.4 of 3GPP TS 23.380 [2], when the EPC/5GC NF failure is detected by the P-CSCF serving the terminating UE.

#### 6.6.1.2	Quick restoration procedure of IMS terminating session

Figure 6.6.1.2-1 illustrates the quick restoration procedure of IMS terminating session in the EPC/5GC network below.

Figure 6.6.1.2-1: Quick restoration procedure of IMS terminating session

1-3.	The I-/S-CSCF in the terminating network receives an incoming INVITE request and routes it to the P-CSCF serving UE B. Then the P-CSCF routes the INVITE request to UE B.

4-5.	The P-CSCF does not receive any response from UE B after some time out, so the P-CSCF determines that the SIP signalling has already attempted the corresponding re-transmissions or retrials. Therefore, the P-CSCF sends a Rx Authorization-Authentication-Request (AAR) with the Specific-Action AVP set as a new value indicating that it's a test to check whether there is a failure in EPC/5GC network.

6-7.	If the PCF/PCRF has identified that the SMF/PGW-C is reachable, then it sends a Gx Re-Auth-Request (RAR) to the SMF/PGW-C according to the received AAR.

8-9.	If the SMF/PGW-C finds that there is no PDU session context of User B, it returns an Rx Re-Auth-Answer (RAA) with the Result-Code AVP set to DIAMETER_UNKNOWN_SESSION_ID (5002) specified in IETF RFC 6733 [12] to the PCF/PCRF.

10.	The PCF/PCRF returns an Rx Authorization-Authentication-Answer (AAA) with the Result-Code AVP set to DIAMETER_UNKNOWN_SESSION_ID (5002) specified in IETF RFC 6733 [12] or the Rx Experimental-Result-Code AVP set to IP-CAN_SESSION_NOT_AVAILABLE (5065) specified in 3GPP TS 29.214 [11] to the P-CSCF.

11-12.	The P-CSCF sends a 503 Service Unavailable response with the P-CSCF Restoration indication to the I-/S-CSCF. Then the I-/S-CSCF starts the HSS based P-CSCF Restoration procedure in EPC network specified in clause 5.4.2.1 of 3GPP TS 23.380 [2], or the UDM/HSS based P-CSCF Restoration procedure in 5GC network specified in clause 5.8.4 of 3GPP TS 23.380 [2].

During the HSS based or UDM/HSS based P-CSCF Restoration procedure, re-attach and IMS registration procedure of UE B will be invoked quickly. Then UE B will receive the next incoming session for it has re-connected to the EPC/5GC network successfully.

### 6.6.2	Impacts on services, entities and interfaces

The requirements for P-CSCF:

-	Support to send an Rx Authorization-Authentication-Request (AAR) with the Specific-Action AVP set as a new value indicating that it's a test to check whether the SMF/PGW-C is normal when timeout.

-	Support to send a 503 Service Unavailable response with the P-CSCF Restoration indication to the I-/S-CSCF after receiving the Rx AAA with the Result-Code AVP set to DIAMETER_UNKNOWN_SESSION_ID (5002) or the Rx Experimental-Result-Code AVP set to IP-CAN_SESSION_NOT_AVAILABLE (5065).

The requirements for S-CSCF:

-	Support to start the HSS based P-CSCF Restoration procedure specified in clause 5.4.2.1 of 3GPP TS 23.380 [2] in EPC network or the UDM/HSS based P-CSCF Restoration procedure specified in clause 5.8.4 of 3GPP TS 23.380 [2] in 5GC network, after receiving a 503 Service Unavailable response with the P-CSCF Restoration indication.

The requirements for PCF/PCRF:

-	Support to send a Gx Re-Auth-Request (RAR) to the SMF/PGW-C after receiving an Rx AAR with the Specific-Action AVP set as ACCESS_NETWORK_INFO_REPORT (12) from the P-CSCF.

-	Support to return a Rx AAA with the Result-Code AVP set to DIAMETER_UNKNOWN_SESSION_ID (5002) or the Rx Experimental-Result-Code AVP set to IP-CAN_SESSION_NOT_AVAILABLE (5065) to the P-CSCF when a response with the Result-Code AVP set to DIAMETER_UNKNOWN_SESSION_ID (5002) is returned by the SMF/PGW-C.

The requirement for SMF/PGW-C:

-	Support to return a Gx RAA with the specific error code to the PCF/PCRF when the SMF/PGW-C is not reachable.

### 6.6.3	Pros

This solution can speed up the restoration procedure of the terminating session when the EPC/5GC NF fails and utilizes the existing HSS based P-CSCF Restoration procedure specified in clause 5.4.2.1 of 3GPP TS 23.380 [2] in EPC network or the UDM/HSS based P-CSCF Restoration procedure specified in clause 5.8.4 of 3GPP TS 23.380 [2] in 5GC network.-.

### 6.6.4	Cons

This solution introduces the extra requirements described in clause 6.6.2 to the SMF/PGW-C, P-CSCF and PCF/PCRF when the terminating UE does not respond to the initial INVITE request and it will increase the system overhead of these NFs.

## 6.7	Solution#7: Enhance the P-CSCF restoration in the case of 5GC/EPC UP failure

### 6.7.1	Description

#### 6.7.1.1	General

The solution is to address the Key Issue #3: "Speed up the restoration procedure of the terminating session when EPC/5GC NF fails".

When the PGW-U or the UPF fails, the current IMS session will fail. On development of network, the 3G network will be taken out of the network and CS fallback will not be supported. Thus the UE will not be informed of such failure. The UE will remain unavailable until it initiates any IMS procedure, i.e. re-registration, or originating IMS session.

In order to inform the UE of CN node failure immediately, the HSS based P-CSCF Restoration procedure specified in 3GPP TS 23.380 [2] can be re-used with some enhancement of notifying CN user plane (e.g. PGW-U/UPF) failure.

#### 6.7.1.2	CN user plane failure detected

The below Figure 6.7.1.2-1 illustrates ICMP message to notify CN user plane (e.g. PGW-U/UPF) failure.

Figure 6.7.1.2-1: CN user plane failure detected during the IMS terminating session

1-3.	The S-CSCS in the originating network sends SIP INVITE to the I/S-CSCF in the terminating network. The I/S-CSCF routes the INVITE request to the P-CSCF in the terminating network.

4.	The P-CSCF in terminating network sends SIP INVITE as the DL data to the PGW-U/UPF.

5-6.	If there is no valid UE context in the PGW-U/UPF (e.g. because of network node restart) or any UP failure is detected by the PGW-U/UPF, the PGW-U/UPF cannot forward any DL data including SIP INVITE to the UE. Based on the operator’s policy, the PGW-U/UPF detects that the data delivery failure is for the IMS related messages. The PGW-U/UPF sends the ICMP Destination Unreachable message to the P-CSCF.

The Type = 3 included in the ICMP header indicates that it is an ICMP message representing Destination Unreachable as specified in IEFT RFC 777[15]. The code included in the ICMP header indicates the detailed reason, which can be utilized by the operator for IMS failure.

NOTE 1:	The PGW-U/UPF may detect IMS related messages based on Deep Packet Inspection (DPI) mechanism of data detection. IMS service is one of the most important services for operators. Thus operators may configure some reserved IP addresses and ports to transfer IMS service. The PGW-U/UPF may identify IMS related messages based on these reserved addresses and ports.

6a.	The P-CSCF detects IMS message transfer failure based on the code value (specified by operator) in ICMP header.

7.	On detecting IMS message transfer failure indicated in ICMP Destination Unreachable message, P-CSCF decides to send a response with an error code (i.e. the UE registration data is not present) to the S-CSCF or not send any response. The S-CSCF shall identify whether the called UE's terminating P-CSCF is not able to process this request, based on received error codes (i.e. the UE registration data is not present) or no response. Then the HSS based P-CSCF restoration as specified in clause 5.4.2.1 in 3GPP TS 23.380 [2] is invoked.

NOTE 2:	There may be different cases of DL data forwarding failure, e.g. RAN node failure, UE unreachable, CN congestion, and etc. The PGW-U/UPF cannot differentiate these cases. Although the P-CSCF restoration is invoked, the restoration may not be performed successfully because the signaling may not be transferred to the UE e.g. in the case of RAN failure or CN congestion. The procedure should not increase the congestion situation when CN congestion happens.

### 6.7.2	Impacts on services, entities and interfaces

The enhancements for PGW-U/UPF:

-	Send an ICMP Destination Unreachable message with the operator specific code value if the IMS related messages cannot be transferred to the UE.

The enhancements for P-CSCF:

-	Detect IMS message transfer failure based on the operator specific code value included in the ICMP Destination Unreachable message.

-	Consider the CN failure cases indicated by PGW-U/UPF as P-CSCF failure and trigger the HSS based P-CSCF restoration.

### 6.7.3	Pros

This solution can speed up the restoration procedure of the terminating session when the CN UP (i.e. P-GW-U or UPF) fails and utilizes the existing HSS based P-CSCF Restoration procedure specified in 3GPP TS 23.380 [2].

This solution re-uses the ICMP message to detect the CN UP failure. It avoids modifying the functionality defined in 3GPP.

### 6.7.4	Cons

This solution enhances implementation logic on IP layer of the P-CSCF and PGW-U/UPF.

## 6.8	Solution#8: Enhance the P-CSCF restoration in the case of 5GC/EPC failure

### 6.8.1	Description

#### 6.8.1.1	General

The solution is to address the Key Issue #3: "Speed up the restoration procedure of the terminating session when EPC/5GC NF fails".

When PGW-C/SMF or PGW-U/UPF fails, the current IMS session will fail. On development of network, the 3G network will be taken out of the network and CS fallback will not be supported. Thus the UE will not be informed of such failure. The UE will remain unavailable until it initiates any IMS procedure, i.e. re-registration, or originating IMS session.

In order to inform the UE of CN node failure immediately, the HSS based P-CSCF Restoration procedure specified in 3GPP TS 23.380 [2] can be re-used with some enhancement of notifying CN node (e.g. PGW-C/SMF) failure.

#### 6.8.1.2	PCRF/PCF detecting PGW-C/SMF failure

The below Figure 6.8.1.2-1 illustrates procedure that the PCRF/PCF detects and notifies the PGW-C/SMF failure.

Figure 6.8.1.2-1: PCRF/PCF detecting PGW-C/SMF failure during the IMS terminating session

1-3.	The S-CSCF in the originating network sends SIP INVITE to the I/S-CSCF in the terminating network. The I/S-CSCF routes the INVITE request to the P-CSCF in the terminating network.

4-5.	When the P-CSCF (acting as the AF) determines that session related media requires PCC supervision, or the access network information (i.e. user location and/or user timezone information) is required, the P-CSCF in terminating network sends a Rx Authorization-Authentication-Request (AAR) to the PCRF as specified in TS 29.214 and the PCRF returns a Rx Authorization-Authentication-Answer (AAA) to the P-CSCF.

6-7.	The PCRF/PCF decides to detect the availability of CN nodes. PCRF sends a Gx Re-Auth-Request (RAR) to the PGW-C/SMF for availability detection and receives no response if the PGW-C/SMF fails. If the PGW-C/SMF detects some errors in user plane, it may send a response with service unavailability indication.

If PCF and SMF are utilized, the Npcf_SMPolicyControl_UpdateNotify is invoked. When the entire SMF set fails, each response returned from the SMF in the SMF set indicates the service unavailability or there is no response received.

NOTE:	The PCRF/PCF decides to detect the availability of CN nodes based on the operator’s policy configuration. Frequent CN node failure detection shall be avoided. For example, the PCRF/PCF is configured to count the number of Rx sessions associated with the same PGW-C/SMF or the PCRF/PCF triggers the detection periodically.

8.	The CN nodes failure is detected in above step, PCRF/PCF sends Rx Abort-Session-Request (ASR) to the P-CSCF including the service unavailable indication. The PCRF/PCF may store the failure status of the CN failure related to the same PGW-C or the same SMF set locally for future usage e.g. to avoid triggering Gx RAR message in each time. If any request is received from the PGW-C or any SMF in the SMF set, the PCRF/PCF will update the status of the PGW-C or the SMF set to available.

9.	Based on this indication, P-CSCF decides to send a response with an error code (i.e. the UE registration data is not present) to the S-CSCF or not send any response. The S-CSCF shall identify whether the called UE's terminating P-CSCF is not able to process this request, based on received error codes (i.e. the UE registration data is not present) or no response. Then the HSS based P-CSCF restoration as specified in clause 5.4.2.1 in TS23.380 is invoked.

### 6.8.2	Impacts on services, entities and interfaces

The enhancements for PCRF/PCF:

-	Decide to initiate the Gx RAR to the PGW-C or invoke Npcf_SMPolicyControl_UpdateNotify to the SMF in the SMF set based on operator’s policy on receiving Rx AAR from P-CSCF.

-	Send the Rx ASR to indicate CN node failure, if PGW-C/SMF or PGW-U/UPF failure is detected.

-	Store the CN nodes failure status for future usage e.g. to avoid triggering Gx RAR message in each time.

The enhancements for PGW-C/SMF:

-	Indicate UP node failure in the Gx RAA or Npcf_SMPolicyControl_UpdateNotify response.

The enhancements for P-CSCF:

-	Consider the CN failure cases indicated by the PCRF/PCF as P-CSCF failure and trigger the HSS based P-CSCF restoration on receiving the service unavailable indication from the PCRF/PCF.

### 6.8.3	Pros

This solution can speed up the restoration procedure of the terminating session when the CN nodes (i.e. PGW-C/SMF, PGW-U/UPF) fails and utilizes the existing HSS based P-CSCF Restoration procedure specified in 3GPP TS 23.380 [2].

This solution enables the operator to configure the detection frequencies to avoid frequent detection to the PGW-C/SMF.

### 6.8.4	Cons

This solution extends the messages exchanged between the PCRF/PCF and the P-CSCF, and between the PCRF/PCF and the PGW-C/SMF.

This solution requires operator to configure the proper detection period. It is difficult to make balance between heavy load triggered by the detection and detecting failure in time. Too frequent detection which may bring more load to the CN should be avoided. However, the long periodic may cause the failure cannot be detected in time.

## 6.9	Solution #9: Solution for speeding up the restoration procedure of the terminating session when EPC/5GC NF fails and EPC/5GC initiate the restoration procedures

### 6.9.1	Description

#### 6.9.1.1	General

The solution addresses Key Issue #3 "Speed up the restoration procedure of the terminating session when EPC/5GC NF fails."

The existing Solution #6, introduces a new Packet Core "Health-Check" procedure, triggered by P-CSCF on various failure scenarios, e.g., the failure to receive an answer for the SIP INVITE, due to a UPF/PGW-U failure.

This solution, builds on Solution #6, by:

-	describing the additional case in which 5GC/EPC can initiate the recovery procedure by itself, instead of just reporting the fault and waiting for IMS to initiate the P-CSCF Restoration.

-	elaborate on the outcome of the Packet Core health Check procedure, and on the need to propagate this outcome to other IMS NFs for statistics/ fault reporting.

The solution description below focusses on the case of the UPF/PGW-U failure. However other situation may apply.

The existing 5GC/EPC procedures allow SMF/PGW-C to detect the UPF/PGW-U failure through the PFCP Heartbeat Request and Heartbeat Response mechanism. SMF/PGW-C can then start restoring the PDU Sessions served by the faulty UPF/PGW-U. However the complete restoration of the large number of PDU session can take a long time (e.g. 30 min), and therefore many UEs will be unreachable for the terminating calls during this time (assuming the affected UEs do not initiate UL data transmission procedures or originating calls).

With the proposed solution, the SMF/PGW-C can prioritize the PDU Session recovery for the UE subject to the packet core health-check procedure, if it is aware of the UPF/PGW-U failure.

The solution is more network efficient because it achieves the same outcome as the IMS P-CSCF Restoration procedure, without involving IMS (S-CSCF, HSS/UDM).  Please note the P-CSCF restoration involve a PDU Session restoration as the key part of the procedure.

#### 6.9.1.2	Quick restoration procedure for IMS Terminating session initiated by the 5GC/EPC

Figure 6.9.1.2-1 depicts the quick restoration procedure initiated by 5GC/EPC for the IMS Terminating session. Please note the call flow assumes Rx interface between P-CSCF and PCF/PCRF and the N7 interface between PCF/PCRF and SMF/PGW-U. However this should not preclude the usage of N5, or Gx interfaces.

Figure 6.9.1.2-1 Quick restoration procedure for IMS Terminating session initiated by the 5GC/EPC

1.	Using the PFCP Heartbeat Request and Heartbeat Response monitoring mechanism, SMF/PGW-C detects the UPF/PGW-U failure. As result SMF/PGW-C may start the PDU Session restoration procedure (PDU Session Release with Reactivation Procedure) for the UEs affected by the UPF/PGW-U failure. Since the PDU session recovery for the large number of users takes a long time, the scenario assumes that UE-B PDU session has not yet recovered.

2-4.	I-/S-CSCF in the terminating network receives a call for UE-B and the call is delivered towards UE-B. Since UPF/PGW-U is defective, the INVITE never reaches UE-B.

5-6.	The P-CSCF does not receive any response from UE B during a configurable time (the timer that should allow for at least one SIP retransmission). As result P-CSCF decides to initiate the packet core Health-check procedure by sending PCF/PCRF an Rx Authorization-Authentication-Request (AAR) with the Specific-Action AVP set as a new value.

7.	PCF/PCRF, after identifying that SMF/PGW-C is reachable, sends a N7: Npcf_SMFPolicyControl_UpdateNotify indicating the request to perform the actions related to the new the Health-check procedure.

8.	SMF/PGW-C may already be aware of the UPF/PGW-U failure, and that the PDU Session for the UE-B was not yet restored. Alternatively, SMF/PGW-C may initiate a UPF/PGW-U check, which returns a user plane NF failure. As result, SMF/PGW-U decides to initiate the PDU Session restoration for UE-B.

9.	SMF/PGW-U triggers the PDU session restoration procedure (PDU Session Release with Reactivation Procedure, see 3GPP TS 23.380 [2], clause 5.8.2.4 "PDU Session Release with Reactivation Procedure").

9a.	After the PDU Session reactivation is completed, the UE-B may need to re-register on the IMS network.

10-11.	SMF/PGW-U notifies PCF/PCRF that a PDU Session restoration procedure was triggered as result of the Health-check procedure, by sending an Npcf_SMPolicyControl_Update Request including a “PDU Session Restoration triggered” flag. PCF/PCRF informs P-CSCF about the outcome of the health-check procedure, by sending an Rx Re-Authorization Request (RAR) including the "PDU Session Restoration triggered" flag.

12.	P-CSCF sends a 500 (Server Internal Error) towards the IMS Core including in the Reason Header the outcome of the Health-check procedure, which in this case will indicate the "PDU Session Restoration triggered" by the SMF/PGW-C. The Packet core health-check outcome information received in the Reason Header is used by the other IMS NFs for statistics/ fault reporting.

13.	As described in 3GPP TS 23.380 [2], the I/S-CSCF shall send a SIP response back to the originating side. This shall be an error response if only one Private User Identity is registered, since the S-CSCF is not able to progress the request; otherwise the S-CSCF shall select the best possible response following normal forking procedures. Subject to an operator policy, the error response sent by S-CSCF may in addition inform the originating side that a terminating request reattempt is possible based on timer expiration.

#### 6.9.1.3	Reporting the outcome of the Packet Core Health-Check procedure to IMS Core NFs

After triggering the Packet Core Health-check procedure, P-CSCF may receive three categories of answers:

a)	Success - everything in the Packet Core functions correctly.

b)	Error with restoration action taken by Packet Core - An example of such case is when SMF/PGW-C decides to perform a PDU Restoration procedure described in the previous section.  In this case, there is no Restoration action expected from IMS.

c)	Error with no restoration action taken by Packet Core – For such case an IMS triggered P-CSCF restoration may be needed, as already described in Solution 6

Regardless of the type of answer received by P-CSCF, it should pass this information to the other IMS NF by using the Reason Header in 500 (Server Internal Error) sent to IMS Core. This information is useful for statistics and fault reporting in e.g., S-CSCF or MMTEL-AS. For example, a packet core Health-check success indication is useful for fault reporting / finding since it would help narrowing down the reason for call failure to a Radio Access Network issue (e.g., a UE-B coverage issue)

### 6.9.2	Impacts on services, entities and interfaces

NOTE this section outlines the impacts in addition to those already described in Solution #6

The requirements for P-CSCF:

-	Support sending the outcome of the Packet core Health-check procedure received over Rx/N5 to the IMS Core NFs, by including the information in the Reason Header of the SIP 5xx error answer.

The requirement for SMF/P-GW-C

-	Following the request for a Health-check procedure for a specific user, if SMF/PGW-U is aware of the failure of UPF/PGW-U serving the user, then it shall initiate the PDU Session restoration (PDU Session Release with Reactivation Procedure, see 3GPP TS 23.380 [2], clause 5.8.2.4), and it shall notify PCF/PCRF about the fact that a restoration procedure was initiated.

### 6.9.3	Pros

The solution minimizes the network resource usage by having the packet core NF responsible for detecting the failure also triggering the related restoration procedure. No additional IMS NF (S-CSCF, HSS/UDM) need to be involved for specific error cases.

Additionally, the solution helps improving the IMS fault detection and reporting by proposing to relay the outcome of the packet core health-check procedure to the other IMS Core NFs

### 6.9.4	Cons

This solution introduces the extra requirements described in clause 6.9.2 to the P-CSCF and SMF/PGW-C when the terminating UE does not respond to the initial INVITE request, and it will increase the system overhead of these NFs.

## 6.10	Solution #10: Dynamic extension of registration expiration timers by the Registration Management function

### 6.10.1	Description

#### 6.10.1.1	General

The solution is to address Key Issue #4: "How to reduce the load of HSS to prevent IMS disaster when the HSS overloads".

The solution consists of enhancing SBI overload control specified in 3GPP TS 29.500 [14] and Diameter overload control specified in IETF RFC 7683 [8] to introduce a new traffic abatement algorithm that allows the network to dynamically extend the registration expire timers as an action to mitigate the HSS/UDM overload and minimize the risk for registered users to became deregister again upon registration refresh attempt while the HSS/UDM remains overloaded. This solution allows the registered users to preserve the registration status and access the IMS services. This solution will also contribute to speed up the recovery time.

The UE and the IMS services will remain unavailable until a Registration to a mobile access network 4G or 5G and IMS network is completed successfully.

Usually, to recover from severe HSS/UDM overload it takes longer time than the negotiated 5GS periodic registration refresh time in 5G access, than the negotiated tracking area update timer in 4G or the negotiated registration expire timer in IMS.

When HSS/UDM are overloaded might invoke cooperative overload protection control by reporting congestion status to network peers in the downstream direction and order upstream traffic reduction to mitigate the overload and facilitate faster recovery.

While HSS/UDM is overloaded and congestions mechanisms are enforced in the network, registered users can become deregistered again due to rejection of registration and due to registration timeouts and therefore will not be able to establish IMS sessions. Both initial registration and re-registration within the recovery time window will compete on the same available resources in an overloaded HSS/UDM.

The solution enables the operator to activate a mechanism to prevent the deregistration of the UEs that managed to successfully register or re-register while the HSS or UDM is overloaded.

The solution prevents the registered UEs to re-enter the network while the HSS/UDM remains overloaded and risk to became deregistered and lose the access to the IMS services.

#### 6.10.1.2	Extension of registration expiration timers upon reception of congestion reports from HSS/UDM via Overload Control Information (OCI) HTTP header

The current SBI overload control specified in 3GPP TS 29.500 [14] can be enhanced to include parameters that allows NF Service Provider to indicate the NF Service Consumer that the extension registration timer traffic abatement algorithm has to be applied. In bold are indicated new parameters to be added in 3gpp-sbi-oci header.

;

; Header: 3gpp-Sbi-Oci

;

Sbi-Oci-Header  = "3gpp-Sbi-Oci:" OWS oci-element *( OWS "," OWS oci-element ) OWS

oci-element     = timestamp ";" RWS validityPeriod ";" RWS olcMetric ";" RWS olcScope

";" RWS olcExtRegTimer

timestamp       = "Timestamp:" RWS DQUOTE date-time DQUOTE

validityPeriod  = "Period-of-Validity:" RWS 1*DIGIT "s"

olcMetric       = "Overload-Reduction-Metric:" RWS ( "100" / %x31-39 DIGIT / DIGIT ) "%"

olcExtRegTimer	= "Registration-Extension-Timer:" RWS registration-timer-value

olcScope        = nfProducerScope / nfConsumerScope / scpScope / seppScope

nfProducerScope = (   ( "NF-Instance:" RWS nfinst )

/ ( "NF-Set:" RWS nfset )

/ ( "NF-Service-Instance:" RWS nfservinst [ ";" RWS "NF-Inst:" RWS nfinst ] )

/ ( "NF-Service-Set:" RWS nfserviceset )

) [ ";" RWS sNssaiList ";" RWS dnnList ]

nfConsumerScope = ( "NFC-Instance:" RWS nfinst [ ";" RWS "Service-Name:" RWS servname ] )

/ ( "NFC-Set:" RWS nfset [ ";" RWS "Service-Name:" RWS servname ])

/ ( "NFC-Service-Instance:" RWS nfservinst [";" RWS "NF-Inst:" RWS nfinst ] )

/ ( "NFC-Service-Set:" RWS nfserviceset )

/ ( "Callback-Uri:" RWS DQUOTE URI DQUOTE *( RWS "&" RWS DQUOTE URI DQUOTE ))

servname        = token

scpScope        = "SCP-FQDN:" RWS fqdn

seppScope       = "SEPP-FQDN:" RWS fqdn

fqdn            = token

dnnList         = "DNN:" RWS 1*tchar *( RWS "&" RWS 1*tchar )

sNssaiList      = "S-NSSAI:" RWS snssai *( RWS "&" RWS snssai )

snssai          = 1*tchar

registration-timer-value = "true" / "false"

The below Figure 6.10.1.2-1 illustrates the procedure that the NF Consumer, handling the Registration Management operation, to extends the registration expiration timers, as a new abatement action based on OCI Header overload report in the SBI interface.

Figure 6.10.1.2-1 Extend registration-expiration-timer abatement action based on OCI Header

1-2. The UE sends Initial Registration or Re-registration to the NF Service Consumer that handles Registration management function, e.g., AMF, MME, P-CSCF/I-CSCF/S-CSCF. The AMF, MME, and I-CSCF/S-CSCF will route the registration queries (e.g. CxQuery) corresponding to (re-)registration procedures call flow, towards the HSS/UDM.

3-4. 	When the HSS/UDM is in overload, the registration request is answered with NF_CONGESTION error code. The answer message will also include the OCI header with the Registration-Extension-Time parameter are intended to inform the NF Service Consumer that the server cannot handle the current received traffic rate, so it shall abate the traffic sent to the NF Service Producer by throttling part of this traffic locally at the NF Service Consumer, or diverting it to an alternative destination (another NF Service Producer where an alternative resource exists) that is not overloaded.

NOTE:	The NF consumer can additionally use traffic abatement and enhancements proposed in next step during procedures other than (re-)registration (e.g. terminating requests resulting in Cx-LocationQuery by I-CSCF).

In step 4, the registration request is rejected by the NF Service Consumer (AMF/MME/I-CSCF/S-CSCF) with the correspondent error.

5.     NF Service Consumer will apply the abatement algorithms included in OCI Header sent by NF Service Provider. Legacy abatement algorithm (throttling) is enhanced with extend the registration timer abatement algorithm as the Registration-Extension-Time parameter is included in the OCI Header. The validity period, also included in the OCI Header, will provide NF Service Consumer about the estimated period of the abatement algorithms applicability.

6-9. Any new initial registration or re-registration procedure would be impacted by the legacy overload protection mechanisms (throttling) if they were also selected by the NF Service Provided in the OCI Header included in the answer provided in step 3. When the NF Service Consumer receives any success answer from NF Service Provided to any initial registration or re-registration procedure while the validity period received in the OCI Header is still applicable, the registration answer towards the UE will include the value of the extended registration timer as registration timer instead of the default registration timer, the extended registration time will be increased beyond the validity period value provided in the OCI header, preventing the reception of a new re-registration request from this UE during the estimated time the NF Service Producer will be overloaded.

NOTE:	Based on local policies, the I-CSCF/S-CSCF can extend the registration timer even if the Registration-Extension-Time parameter was not included in the OCI header.

10. When the validity time expires without any new overload indication from NF Service Provider or when the NF Service Provider sent a message with an OCI Header indicating that the overload situation ceased (Overload Reduction Metric = 0), the NF Service Consumer restore the registration timer to its default value.

11-14. Once the overload situation finished in the NF Service Provider, any new initial registration or re-registration initiated from UE is answered from NF Service Consumer including the default registration timer.

#### 6.10.1.3	Extension of registration expiration timers upon reception of congestion reports from HSS/UDM via HTTP status codes

The below Figure 6.10.1.3-1 illustrates the procedure that the NF Consumer, handling the Registration Management operation, to extends the registration expiration timers, as a new abatement action based on HTTP status codes in the SBI interface.

Figure 6.10.1.3-1 Extend registration-expiration-timer abatement action based on HTTP status

1-2. The UE sends Initial Registration or Re-registration to the NF Service Consumer that handles Registration management function, e.g., AMF, MME, P-CSCF/I-CSCF/S-CSCF. The AMF, MME, and I-CSCF/S-CSCF will route the registration queries (e.g. Cx-Query) corresponding to registration procedures call flow, towards the HSS/UDM.

3-4. 	When the HSS/UDM is in overload, the registration request is answered with NF_CONGESTION error code. The answer message HTTP status codes (503 and 429) are intended to inform the NF Service Consumer that the server cannot handle the current received traffic rate.

In step 4, the registration request is rejected by the NF Service Consumer (AMF/MME/I-CSCF/S-CSCF) with the correspondent error).

5.     When the number of HTTP status code indicating congestion crosses a predefined value within a time frame in NF Service Consumer, it will start applying the registration extension timer.

NOTE:	Based on local policies, the I-CSCF/S-CSCF can extend the registration timer even if the Registration-Extension-Time parameter was not included in the OCI header.

6-9. For any new initial registration or re-registration procedure which is not answered by NF Service Provider with an HTTP status code indicating congestion, the registration answer towards the UE will include the value of the extended registration timer as registration timer instead of its default value. It will prevent the reception of a new re-registration request from this UE during the estimated time the NF Service Producer will be overloaded.

10. When the number of HTTP status code indicating congestion received from NF Service Provider goes under a predefined threshold within a time frame, the NF Service Consumer will resume the usage of the default registration timer in any new registration request from any UE.

11-14. Once the NF Service Consumer considers that the overload situation is finished in the NF Service Provider, any new initial registration or re-registration initiated from UE is answered from NF Service Consumer including the default registration timer.

#### 6.10.1.4	Extension of expiration timers upon reception of congestion reports from HSS/UDM via Diameter Overload Indication Conveyance (DOIC) information

The current Diameter Overload control described in IETF RFC 7683 [8] needs to be enhanced to include the Extend registration expiration timer abatement algorithm in the DOIC information. New values to be supported in the correspondent AVP are indicated in Figure 6.10.1.4-1. The proper value has to be provided by IANA, value 8 has been considered as an example in the following figure.

Figure 6.10.1.4-1 Diameter DOIC information enhancement

The below Figure 6.10.1.4-2 illustrates the procedure that the Diameter Server (HSS/UDM), handling the Registration Management operation, to extends the registration expiration timers, as a new abatement action based in DOIC information provided by the HSS/UDM in the diameter interface.

Figure 6.10.1.4-2 Extend registration-expiration-timer abatement action based Diameter DOIC information

1-4. When the Diameter Server (HSS/UDM) is in overload situation, the registration request from Diameter Client (S-CSCF/MME) is answered by HSS/UDM with Diameter_too_busy error code, the answer message will also include the DOIC information selecting Extend-registration-expiration-timer abatement algorithm and optional any legacy abatement algorithm (throttling).

5-9. Any new initial registration or re-registration procedure would be impacted by the legacy overload protection mechanisms (throttling) it they were also selected by the Diameter Server in the DOIC information included in the answer provided in step 3. When the Diameter Client receives any success answer from HSS/UDM to any initial registration or re-registration procedure, the registration answer towards the UE will include the value of the extended registration timer as registration timer instead of the default registration timer, the extended registration time will be increased beyond the validity duration value provided in the DOIC information, preventing the reception of a new re-registration request from this UE during the estimated time the Diameter Server will be overloaded.

10. When the validity duration time expires without any new overload indication from the Diameter Server or when the Diameter Server sent a message with an DOIC information indicating that the overload situation ceased (OC-Reduction-Percentage = 0), the Diameter Client restore the registration timer to its default value.

11-14. Once the overload situation finished in the Diameter Server, any new initial registration or re-registration initiated from UE is answered from Diameter Client including the default registration timer.

### 6.10.2	Impacts on services, entities and interfaces

Enhancements common for NF Service Producer (e.g HSS/UDM) and NF Service Consumer (AMF, MME, S-CSCF) in the case of congestion handling using OCI header or HTTP status code:

-	The current SBI overload control specified in 3GPPS TS 29.500 [14] needs to be enhanced to include parameters that allows NF Service Provider (e.g HSS/UDM) to indicate the NF Service Consumer (AMF, MME, S-CSCF) that the extension registration timer traffic abatement algorithm has to be applied.

The enhancements for NF Service Consumer (e.g MME/S-CSCF) and NF Service Provider (e.g HSS/UDM) , in the case of congestion handling using DOIC information over diameter:

-	The current Diameter Overload control described in IETF RFC 7683 [8] needs to be enhanced to include the Extend registration expiration timer abatement algorithm in the DOIC information.

The enhancements for NF Service Consumer (e.g AMF/MME/S-CSCF):

-	AMF, MME and S-CSCF to dynamically modify and extend the Registration timer.

-	AMF, MME and S-CSCF to Dynamically restore the Registration timer.

### 6.10.3	Pros

This solution allows the new abatement algorithm to be combined with the existing overload abatement algorithms e.g throttling specified in 3GPP TS 29.500 [14] and allows the combination with the algorithms (default) specified in IETF RFC 7683 [8] or the overload rate control Diameter overload control specified in IETF RFC 8582 [13].

The proposed enhancement to DOIC allows the reporting node to select and activate a legacy abatement algorithm in combination to the new abatement algorithm “Extend-registration-expiration-timers", allowing the following abatement algorithms new combinations (c, d, e):

a)	loss (defaultl)

b)	rate controlc)	Extend-registration-expiration-timersd)	loss (default) + Extend-registration-expiration-timerse)	rate control + Extend-registration-expiration-timers

This solution reduces the frequency of re-registration requests from end users while reducing in this way the number of messages towards the HSS or/and UDM. This solution reduces the overload in the HSS/UDM and speed up the recovery.

The solution enables the operator to dynamically activate a mechanism to prevent the deregistration of the UEs that managed to successfully register or re-register while the HSS or UDM is overload and congestion control mechanism are activated in multiple network functions.

The solution prevents the registered UEs to re-enter the network while the HSS/UDM remains overloaded and risk to became deregistered and loose access to the IMS services.

### 6.10.4	Cons

The process to extend the Diameter Overload Indication Conveyance (DOIC) requires the registration of the new abatement algorithm in IANA, requiring the "expert review" process by IETF of a publicly available specification (e.g. a 3GPP Technical Specification).

# 7	Evaluations

## 7.1	Evaluation of Solutions for Key Issue#1

A group of Solutions have been proposed for Key Issue#1 to address the IMS (re-)registration and MT services interruption for UEs due to HSS failure or overload.

Solution #1 ensures successful IMS re-registration of a registered UE in the IMS network when HSS fails or overloads. This prevents the failure of IMS re-registration from causing the UE to initiate an initial registration and accelerate the formation of the signalling storm.

In the solution for the re-registration process without I-CSCF, P-CSCF forwards the REGISTER request directly to the S-CSCF to enable re-registration bypassing the HSS while minimally altering the existing network functionality.

The solution of the re-registration procedure for routing with I-CSCF adds the S-CSCF address stored in P-CSCF to a new header field of the REGISTER request. Then, the I-CSCF forwards the REGISTER request to the S-CSCF bypassing the HSS by parsing the S-CSCF address of the new header field.

Solution#3 proposes a new AS to guarantee the IMS initial registration and MT services continue for UEs registered to the IMS network in case of HSS failure or overload, which supplements Solution #1. UE registration-related data (e.g. S-CSCF address, IMSI, MSISDN, etc.) of the new AS are synchronously derived from the ISC interface (S-CSCF-AS) directly or indirectly. The new AS not only supports directly forwarding the IMS REGISTER/INVITE requests to the S-CSCF for Option A, but also supports the return of the 305 (Use Proxy) with the S-CSCF address to the I-CSCF, which then sends the IMS REGISTER/INVITE requests to the S-CSCF serving UEs for Option B. Option A and Option B don't require any modification of existing network functions. There are fewer signalling interactions for REGISTER/INVITE procedures using Option A. This increases the flexibility of new AS applications. The solution supports IMS (re-)registration and MT services from roaming or non-roaming UEs. But Solution#3 introduces a new AS which needs to work even in the normal scenario. The recovery mechanism of the new AS needs to be considered.

Solution#4 proposes to utilize the OPTIONS mechanism to find the S-CSCF serving the subscriber for bypassing HSS, requiring the S-CSCF to delay removing all subscription information related to this specific Public User Identity in the de-registration procedure. However, Solution#4 introduces a new CSCF interaction mechanism, the significant signalling processing overhead of the I/S-CSCF, the overhead of S-CSCF delay removing all subscription data, and the increased SIP session latency (especially for calls) need to be considered.

Table 7.1-1 shows the comparison of Solution#3 and Solution#4 as follows. Both Solution#3 and Solution#4 has an assumption that there are available AVs of the subscriber in the S-CSCF.

Table 7.1-1 Comparison of Solution#3 and Solution#4

|  | Solution#3 | Solution#4 |
| --- | --- | --- |
| Pros | Support IMS registration and MT call service of subscribers in case of HSS/UDM failure or overload. | Support IMS registration and MT call service of subscribers in case of HSS/UDM failure or overload. No impact to the existing IMS signalling procedure in normal case. |
| Cons | - Network deployment and adjustment is needed for the dedicated AS. E.g., the interconnection with surrounding nodes and OAM management. - Modifications to the shared iFC in S-CSCF are needed if the shared iFC is deployed in the operator’s network. If shared iFC is not deployed, all the UE subscription data in HSS need to be modified for the iFC data which triggered to the new AS. - S-CSCF needs to trigger the dedicated AS via iFC, which increases the overload of S-CSCF and network traffic in existing UE initial registration procedures in the normal case. - It introduces a new failure point into the network. | - I-CSCF and S-CSCF upgrade are needed for an OPTIONS mechanism used in HSS failure case. - I-CSCF needs to interact with multiple S-CSCFs and in the worst case I-CSCF need to interact with every S-CSCFs. In this case, I-CSCF needs to utilize some mechanism to improve the efficiency when interacting with multiple S-CSCFs. Otherwise, it will significantly increase the system overhead. - S-CSCF needs to delay removing all subscription information related to the specific Public User Identity in the de-registration procedure in HSS failure or overload case. |

If HSS fails, the re-attach or re-registration procedures of the UE in the PS domain cannot be completed. Therefore, both Solution#3 and Solution#4 cannot support the IMS initial registration procedure for deregistered UEs in the scenario of KI#1.

## 7.2	Evaluation of Solutions for Key Issue#2

Solution #5 is the only solution addressing the Key Issue #2: Handle all PCRF/PCF instances failure scenario with minimum impact to the registered IMS users.

Solution #5 proposes a mechanism to bypass the PCRF/PCF to continue the default IMS bearer establishment, the IMS originating session and the IMS terminating session procedure when all the PCRF/PCF instances fail. It introduces the extra requirements to the PGW-U/UPF, PGW-C/SMF, P-CSCF and IMS-AGW and will increase the system overhead of these NFs. It only continues the QoS procedure when all the PCRF/PCF instances fail, but does not support other service scenarios such as Network Provided Location Information, which needs interaction with the PCRF/PCF.

## 7.3	Evaluation of Solutions for Key Issue#3

Solution #6~#9 address the Key Issue #3: Speed up the restoration procedure of the terminating session when EPC/5GC NF fails.

Solution #6 proposes a check mechanism over Rx and Gx interface in the EPC/5GC network for the P-CSCF to distinguish the terminating session failure due to the EPC/5GC NF abnormal and the P-CSCF restores this kind of failure through the HSS based P-CSCF Restoration procedure specified in clause 5.4.2.1 of 3GPP TS 23.380 [2] in EPC network or the UDM/HSS based P-CSCF Restoration procedure specified in clause 5.8.4 of 3GPP TS 23.380 [2]. It introduces the extra requirements to the SMF/PGW-C, P-CSCF and PCF/PCRF.

Solution #7 proposes that once the PGW-U/UPF detects the delivery failure of IMS related messages to a specific UE, it sends an ICMP message indicating destination unreachable to the P-CSCF serving the UE to trigger the HSS based Restoration procedure. It utilizes the Deep Packet Inspection (DPI) mechanism of data detection and the PGW-U/UPF needs to detect the downlink SIP traffic to all the UEs which will increase the system overhead of the PGW-U/UPF. The delivery failure of IMS related messages to a specific UE may be caused by different cases, but it is unnecessary for the PGW-U/UPF to differentiate these cases. However, re-using the lower layer mechanisms avoids modifying the 3GPP network functionalities. Upon detection of this failure, the P-CSCF invokes the P-CSCF Restoration procedure specified in 3GPP TS 23.380 [2].

Solution #8 proposes the same mechanism as Solution #6 that the NF finding the EPC/5GC failure will execute EPC/5GC availability detection with RAR/RAA procedure and initiate the HSS based or UDM/HSS based Restoration procedure when receiving a response with a specific error code. The only difference between Solution #6 and Solution #8 is the NF which finds the EPC/5GC failure and performs the mechanism. It is the P-CSCF in Solution #6, and the PCF/PCRF in Solution #8 for the failure is founded in different steps.

The procedures of provisioning of service information at terminating P-CSCF and PCRF for EPS is specified in clause B.2.2 of 3GPP TS 29.213 [16] and provisioning of service information at terminating P-CSCF and PCF for 5GS is specified in clause B.2.2 of 3GPP TS 29.513 [17]. There are two options for interaction between P-CSCF and PCRF/PCF depending on the timing of SDP offer (included in INVITE message) as specified in Figure B.2.2-1 of TS 29.213 [16] and TS 29.513 [17] and Figure B.2.2-2 of 3GPP TS 29.213 [16] and 3GPP TS 29.513 [17]. The following improvement for the former case may speed up the restoration:

1. Step 6 is invoked without step 3-5(for both normal case and failure case).

2. The P-CSCF may re-sent Rx AAR to the PCRF or Npcf_PolicyAuthorization_Modify to the PCF upon reception of SDP answer from the UE (normal case).

Solution #9 proposes the same failure check mechanism as Solution #6 that the P-CSCF starts a health check over Rx/N5 and Gx/N7 interface in the EPC/5GC network to distinguish the terminating session failure due to the UPF/PGW-U failure. Moreover, Solution #9 enhances Solution #6 that the SMF/PGW-C can either perform the PDU Restoration procedure directly when it is aware of the UPF/PGW-U failure, or return an Rx Re-Auth-Answer (RAA) with the Result-Code AVP set to a specific value indicating the P-CSCF to perform the HSS based P-CSCF Restoration procedure.

## 7.4	Evaluation of Solutions for Key Issue#4

The signalling traffic to the HSS needs to be reduced when the S-CSCF or HSS identifies that the HSS is in an overload situation by indicators analysis means described in 3GPP TS 29.228 [5] Annex H and IETF RFC 7683 [8] or other internal mechanisms of S-CSCF.

Solution #2 proposes to reduce the burden on the HSS during the IMS initial registration process and the IMS third-party registration process by utilizing available AVs and user profile stored on the S-CSCF in case of HSS overload. This helps reduce the HSS load further decrease the probability of signalling storm and accelerate failure restoration for the IMS network while making minimal changes to the network function. As this would provide benefit to the operation of each network operator, introducing the capabilities for Solution #2 should be considered.

Solution #10 proposes to reduce the signalling by adjusting the re-registration timer returned to UEs based on dynamic conditions in HSS. This allows to alleviate the burden of handling future (and possibly massive) re-registrations (Cx-query) and potential re-authentications (Cx-AuthDataReq) in HSS until the overload situation is mitigated or finished. Given that the signalling would be throttled by I-CSCF/S-CSCF in normal conditions anyway (that is, with normal re-registration timers), HSS may instruct the network to take one step further and require the signalling decrease to the source (UEs). Re-registration timer will then be restored to its original/default value. This solution impacts S-CSCF (dynamic adjustment based on OCI header) and HSS (OCI header is enhanced so that HSS instruct all I-CSCF/S-CSCF in a central manner about dynamic registration timer adjustment under severe overload conditions).

# 8	Conclusions

## 8.1	Conclusion of Solutions for Key Issue#1

For the IMS re-registration procedure, solution#1 is proposed to be standardized.

For the IMS initial registration and IMS MT procedure, both Solution#3 and Solution#4 are proposed to be standardized.

Editor's Note: Based on LS response from SA2 in C4-250034 normative specification work on solutions #1, #2, #3 and #4 cannot proceed for the time being.

## 8.2	Conclusion of Solutions for Key Issue#2

Based on the evaluation in clause 7.2, it is concluded to use Solutions #5 for normative specification work.

## 8.3	Conclusion of Solutions for Key Issue#3

According to the evaluation for Key Issue #3, the following conclusions are concluded:

-	For the failure check, the health check triggered by the P-CSCF described in Solution #6 and #9 or by the PCRF/PCF described in Solution #8 are proposed to be specified in normative work.

-	For the failure restoration, Solution #9 is proposed to be specified in normative work for it is an enhancement to Solution #6.

-	Solution #7 is proposed as an example to detect the P-CSCF failure to be added in 3GPP TS 23.380 [2].

## 8.4	Conclusion of Solutions for Key Issue#4

According to the evaluation for KI#4, reducing the Diameter signalling interactions between S-CSCF/AS and HSS using stored available AVs should be defined and the following conclusions are agreed:

1.	Updates of the IMS registration process in the HSS overload scenario are needed, including:

-	Procedures for IMS initial registration and IMS third-party registration.

2.	Updates of network function enhancements for the IMS registration process in the HSS overload scenario are needed, including:

-	Updates of S-CSCF in 3GPP TS 23.380 [2], and collaborate with SA2 for updating 3GPP TS 23.228 [3] and 3GPP TS 24.229 [6].

It is concluded that solution #2 is selected for normative specification work.

Editor's Note: Based on LS response from SA2 in C4-250034 normative specification work on solutions #1, #2, #3 and #4 cannot proceed for the time being.

It is also concluded that solution #10 is selected for normative specification work, since solution #10 focus on decreasing the traffic towards HSS mainly for Cx-Query and complements solution #2 for Cx-AuthDataReq (for those deployments requiring re-authentication at requests other than initial registration)

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-10 | CT4#118 | C4-234587 | - | - | - | Implementation of C4-234059, C4-234525, C4-234526, C4-234527, C4-234529, C4-234569, C4-234570, C4-234571 and C4-234577 in CT4#118 | 0.1.0 |
| 2023-11 | CT4#119 | C4-235658 |  |  |  | Implementation of C4-235574 and C4-235575 in CT4#119 | 0.2.0 |
| 2024-03 | CT4#121 | C4-240850 |  |  |  | Implementation of C4-240732 and C4-240782 in CT4#121 | 0.3.0 |
| 2024-04 | CT4#122 | C4-241404 |  |  |  | Implementation of C4-241467, C4-241401, C4-241538 and C4-241545 in CT4#122 | 0.4.0 |
| 2024-05 | CT4#123 | C4-242488 |  |  |  | Implementation of C4-242172, C4-242403, C4-242404, C4-242407, C4-242408, C4-242409, C4-242481, C4-242483 and C4-242493 in CT4#123 | 0.5.0 |
| 2024-06 | CT#104 | CP-241240 |  |  |  | Presented for information | 1.0.0 |
| 2024-08 | CT4#124 | C4-243673 |  |  |  | Implementation of C4-243158, C4-243558, C4-243563, C4-243616, C4-243624, C4-243657 and C4-243658 in CT4#124 | 1.1.0 |
| 2024-10 | CT4#125 | C4-244502 |  |  |  | Implementation of C4-244237, C4-244284, C4-244285 and C4-244471 in CT4#125 | 1.2.0 |
| 2025-02 | CT4#127 | C4-250625 |  |  |  | Implementation of C4-250456 in CT4#127 | 1.3.0 |