| 3GPP TS 31.127 V18.3.0 (2025-09) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; UICC-terminal interaction; non-removable Universal Subscriber Identity Module (nrUSIM) application behavioural test specification; (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	24

Introduction	25

1	Scope	26

2	References	26

3	Definitions of terms, symbols and abbreviations	29

3.1	Terms	29

3.2	Symbols	29

3.3	Abbreviations	29

3.4	Coding Conventions	29

3.5	Generic procedures for 5G-NR, E-UTRAN, IMS and NB-IoT	29

3.6	Table of optional features	30

3.7	Applicability	31

3.7.1	Applicability to user equipment	31

3.7.2	Supported additional explicit verification methods	32

3.7.3	Applicability of the individual tests	32

3.8	Applicability table	33

3.9	Rules for the representation of number systems and strings	39

4	Test environment	40

4.1.1	General test environment	40

4.1.2	Example - test environment for implicit testing	41

4.1.3	Example - test environment for contents verification	41

4.1.4	Example - test environment for seamless testing	42

4.1.5	Example – test environment for test toolkit events based testing	42

4.1.6	Example – test environment for UEs with ETSI eSSP/iSSP	43

4.2	Requirements to the EUT and the test environment	43

4.2.1	General Requirements	43

4.2.2	Requirements to the UE (EUT) – supported interfaces	44

4.2.3	Supported RATs	44

4.2.4	Initial and final procedure steps	44

4.3	Random value generation	45

4.4	Suitability assessment	45

4.4.1	Suitability assessment in present test procedures	45

4.4.2	Suitability assessment in test descriptions	45

4.4.3	Suitability assessment in the test protocol	46

4.4.4	Justification of additional measures	46

4.5	Definition of default values	46

4.5.1	Introduction	46

4.5.1.1	Installation, provisioning or modification methods for EFs and DFs	46

4.5.1.2	TS.48 Version and usage	46

4.5.2	Definition of the Default UICC	46

4.5.3	Definition of FDN UICC	47

4.5.4	Definition of E-UTRAN/EPC UICC	48

4.5.5	Definition of E-UTRAN/EPC ISIM-UICC	49

4.5.6	Definition of ACSGL/OCSGL E-UTRAN/EPC UICC	49

4.5.7	Definition of Non-Access Stratum Configuration UICC	56

4.5.8	Definition of Non-Access Stratum Configuration of E-UTRAN/EPC UICC	56

4.5.9	Definition of 5G-NR UICC	56

4.5.10	Definition of 5G-NR UICC – non-IMSI SUPI Type	59

4.5.11	Definition of 5G-NR UICC – support of Rel-16 features	61

4.5.12	Definition of 5G-NR UICC – support of Rel-17 features	63

4.5.13	Definition 5G-NR UICC – CAG support	63

4.5.14	Definition of 5G-NR ISIM-UICC	64

4.5.15	Definition of 5G-NR UICC – support of Rel-18 features	64

4.5.16	Definition of 5G-NR SSIM-UICC	64

4.6	Definition of default values for EFs commonly used with random value generation	65

4.6.1	EFIMSI with 3-digit MNC and random value	65

4.6.2	EFIMSI "short", with 3-digit MNC and random value	65

4.6.3	EFIMSI "short", with 2-digit MNC and random value	66

4.6.4	EFIMSI "long", with 3-digit MNC and random value	66

4.6.5	EFIMSI for SUCI calculation tests	66

4.6.6	EFIMSI for "change IMSI" tests	66

4.6.7	EFRouting_Indicator with 2-digit MNC and random value	67

4.6.8	EFLOCI with "short" random TMSI value	67

4.6.9	EFLOCI with "long" random TMSI value	67

4.6.10	EFEPSLOCI	67

4.6.11	EFFDN	68

4.6.12	EFECC	69

4.7	Default values for NG-SS and UE configurations	69

4.7.1	Default NG-SS and UE configuration for different usage settings	69

4.7.2	Default NG-SS configuration for 3GPP access with IMS service	70

4.7.2.1	Default NG-SS configuration for IMS Registration support	70

4.7.2.2	Default NG-SS configuration for PS voice support	70

4.7.3	Default NG-SS procedure for 5G Registration on 3GPP access with IMS service	70

4.7.4	Default UE configuration for IMS service	70

5	Subscription related tests	70

5.1	IMSI/TMSI handling	70

5.1.1	UE identification by short IMSI	70

5.1.2	UE identification by short IMSI using a 2-digit MNC	70

5.1.3	UE identification by "short" TMSI	70

5.1.4	UE identification by "long" TMSI	71

5.1.5	UE identification by long IMSI, TMSI updating and key set identifier assignment	71

5.1.6	UE identification by short IMSI when accessing E-UTRAN/EPC	71

5.1.6.1	Definition and applicability	71

5.1.6.2	Conformance requirement	71

5.1.6.3	Test purpose	71

5.1.6.4	Method of test	71

5.1.6.4.1	Initial conditions	71

5.1.6.4.2	Procedure	72

5.1.6.4.2.1	Test procedure A	72

5.1.6.4.2.2	Test procedure B	72

5.1.6.5	Acceptance criteria	72

5.1.6.5.1	Acceptance criteria for Test Procedure A	72

5.1.6.5.2	Acceptance criteria for Test Procedure B	73

5.1.6.4.3	Common acceptance criteria	73

5.1.7	UE identification by short IMSI using a 2-digit MNC when accessing E-UTRAN/EPC	73

5.1.7.1	Definition and applicability	73

5.1.7.2	Conformance requirement	73

5.1.7.3	Test purpose	73

5.1.7.4	Method of test	73

5.1.7.4.1	Initial conditions	73

5.1.7.4.2	Procedure	74

5.1.7.4.2.1	Test procedure A	74

5.1.7.4.2.2	Test procedure B	74

5.1.7.4	Acceptance criteria	74

5.1.8	UE identification after changed IMSI with service "EMM Information" not available	75

5.1.8.1	Definition and applicability	75

5.1.8.2	Conformance requirement	75

5.1.8.3	Test purpose	75

5.1.8.4	Method of test	75

5.1.8.4.1	Initial conditions	75

5.1.8.4.2	Procedure	76

5.1.8.4.2.1	Test procedure A	76

5.1.8.4.2.2	Test procedure B	76

5.1.8.5	Acceptance criteria	77

5.1.9	UE identification by GUTI when using USIM with service "EMM Information" not available	77

5.1.10	UE identification by GUTI when using USIM with service "EMM Information" available	79

5.1.10.2	Conformance requirement	79

5.1.10.3	Test purpose	80

5.1.10.4	Method of test	80

5.1.10.4.1	Initial conditions	80

5.1.10.4.2	Procedure	80

5.1.10.4.2.1	Test procedure A	80

5.1.10.4.2.2	Test procedure B	81

5.1.10.4.2.3	Common final Test Procedure	81

5.1.10.5	Acceptance criteria	81

5.2	Access Control handling	82

5.2.1	Access Control information handling	82

5.2.2	Access Control information handling for E-UTRAN/EPC	82

5.2.2.1	Definition and applicability	82

5.2.2.2	Conformance requirement	82

5.2.2.3	Test purpose	82

5.2.2.4	Method of test	83

5.2.2.4.1	Initial conditions	83

5.2.2.4.2	Coding details	83

5.2.2.4.3	Procedure	83

5.2.2.5	Acceptance criteria	83

5.2.3	Access Control information handling for NB-IoT	84

5.2.3.1	Definition and applicability	84

5.2.3.2	Conformance requirement	84

5.2.3.3	Test purpose	84

5.2.3.4	Method of test	84

5.2.3.4.1	Initial conditions	84

5.2.3.4.2	Coding details	85

5.2.3.4.3	Procedure	85

5.2.3.5	Acceptance criteria	85

5.3	Handling subscription identifier privacy for 5G	85

5.3.1	SUCI calculation by ME using null scheme	85

5.3.1.1	Definition and applicability	85

5.3.1.2	Conformance requirement	85

5.3.1.3	Test purpose	86

5.3.1.4	Method of test	86

5.3.1.4.1	Initial conditions	86

5.3.1.4.2	Procedure	87

5.3.1.5	Acceptance criteria	88

5.3.2	SUCI calculation by ME using Profile B	88

5.3.2.1	Definition and applicability	88

5.3.2.2	Conformance requirement	88

5.3.2.3	Test purpose	89

5.3.2.4	Method of test	89

5.3.2.4.1	Initial conditions	89

5.3.2.4.2	Procedure	89

5.3.2.5	Acceptance criteria	90

5.3.3	UE identification by SUCI during initial registration – SUCI calculation by USIM using profile B	90

5.3.3.1	Definition and applicability	90

5.3.3.2	Conformance requirement	90

5.3.3.3	Test purpose	90

5.3.3.4	Method of test	91

5.3.3.4.1	Initial conditions	91

5.3.3.4.2	Procedure	92

5.3.3.5	Acceptance criteria	92

5.3.4	UE identification by SUCI in response to IDENTITY REQUEST message	92

5.3.4.1	Definition and applicability	92

5.3.4.2	Conformance requirement	93

5.3.4.3	Test purpose	93

5.3.4.4	Method of test	93

5.3.4.4.1	Initial conditions	93

5.3.4.4.2	Procedure	94

5.3.4.5	Acceptance criteria	95

5.3.5	UE identification by SUCI in response to IDENTITY REQUEST message with T3519 timer expiry	95

5.3.5.1	Definition and applicability	95

5.3.5.2	Conformance requirement	95

5.3.5.3	Test purpose	96

5.3.5.4	Method of test	96

5.3.5.4.1	Initial conditions	96

5.3.5.4.2	Procedure	97

5.3.5.5	Acceptance criteria	98

5.3.6	UE identification by SUCI in response to IDENTITY REQUEST message and AUTHENTICATION REJECT	98

5.3.6.1	Definition and applicability	98

5.3.6.2	Conformance requirement	98

5.3.6.3	Test purpose	99

5.3.6.4	Method of test	99

5.3.6.4.1	Initial conditions	99

5.3.6.4.2	Procedure	100

5.3.6.5	Acceptance criteria	101

5.3.7	SUCI calculation by the ME using null scheme – missing parameters for subscription identifier privacy support by the USIM	101

5.3.7.1	Definition and applicability	101

5.3.7.2	Conformance requirement	101

5.3.7.3	Test purpose	101

5.3.7.4	Method of test	102

5.3.7.4.1	Initial conditions	102

5.3.7.4.2	Procedure	102

5.3.7.5	Acceptance criteria	102

5.3.8	UE identification by 5G-GUTI – Last Registered TAI stored on USIM	103

5.3.8.1	Definition and applicability	103

5.3.8.2	Conformance requirement	103

5.3.8.3	Test purpose	103

5.3.8.4	Method of test	103

5.3.8.4.1	Initial conditions	103

5.3.8.4.2	Procedure	104

5.3.8.5	Acceptance criteria	104

5.3.9	UE identification by 5G-GUTI – Last Registered TAI stored by ME	105

5.3.9.1	Definition and applicability	105

5.3.9.2	Conformance requirement	105

5.3.9.3	Test purpose	105

5.3.9.4	Method of test	105

5.3.9.4.1	Initial conditions	105

5.3.9.4.2	Procedure	106

5.3.9.5	Acceptance criteria	106

5.3.10	UE identification after SUPI is changed	107

5.3.10.1	Definition and applicability	107

5.3.10.2	Conformance requirement	107

5.3.10.3	Test purpose	107

5.3.10.4	Method of test	107

5.3.10.4.1	Initial conditions	107

5.3.10.4.2	Procedure	108

5.3.10.5	Acceptance criteria	108

5.3.11	SUCI calculation by ME using Profile A	108

5.3.11.1	Definition and applicability	108

5.3.11.2	Conformance requirement	108

5.3.11.3	Test purpose	109

5.3.11.4	Method of test	109

5.3.11.4.1	Initial conditions	109

5.3.11.4.2	Procedure	110

5.3.11.5	Acceptance criteria	110

5.3.12	UE identification by SUCI during initial registration – SUCI calculation by USIM using profile A	110

5.3.12.1	Definition and applicability	110

5.3.12.2	Conformance requirement	110

5.3.12.3	Test purpose	111

5.3.12.4	Method of test	111

5.3.12.4.1	Initial conditions	111

5.3.12.4.2	Procedure	112

5.3.12.5	Acceptance criteria	112

5.3.13	SUCI calculation by ME using null scheme– no Protection Scheme Identifier provisioned in the USIM	113

5.3.13.1	Definition and applicability	113

5.3.13.2	Conformance requirement	113

5.3.13.3	Test purpose	113

5.3.13.4	Method of test	113

5.3.13.4.1	Initial conditions	113

5.3.13.4.2	Procedure	114

5.3.13.5	Acceptance criteria	114

5.3.14	SUCI calculation by ME using null scheme – no Home Network Public Key for supported protection scheme provisioned in the USIM	114

5.3.14.1	Definition and applicability	114

5.3.14.2	Conformance requirement	114

5.3.14.3	Test purpose	115

5.3.14.4	Method of test	115

5.3.14.4.1	Initial conditions	115

5.3.14.4.2	Procedure	115

5.3.14.5	Acceptance criteria	116

5.3.15	SUCI calculation by ME using null scheme with the E-UTRAN/EPC UICC	116

5.3.15.1	Definition and applicability	116

5.3.15.2	Conformance requirement	116

5.3.15.3	Test purpose	116

5.3.15.4	Method of test	116

5.3.15.4.1	Initial conditions	116

5.3.15.4.2	Procedure	117

5.3.15.5	Acceptance criteria	117

5.3.16	SUCI calculation by ME using the lower priority protection scheme when the higher priority protection scheme is not supported by the ME	117

5.3.16.1	Definition and applicability	117

5.3.16.2	Conformance requirement	118

5.3.16.3	Test purpose	118

5.3.16.4	Method of test	118

5.3.16.4.1	Initial conditions	118

5.3.16.4.2	Procedure	119

5.3.16.5	Acceptance criteria	120

5.3.17	SUCI calculation by ME using Profile B with compressed Home Network Public Key	120

5.3.17.1	Definition and applicability	120

5.3.17.2	Conformance requirement	120

5.3.17.3	Test purpose	121

5.3.17.4	Method of test	121

5.3.17.4.1	Initial conditions	121

5.3.17.4.2	Procedure	122

5.3.17.5	Acceptance criteria	123

5.4	Unified Access Control information handling for 5G-NR	123

5.4.1	Unified Access Control – Access identity 0, no access identities indicated by USIM	123

5.4.1.1	Definition and applicability	123

5.4.1.2	Conformance requirement	123

5.4.1.3	Test purpose	124

5.4.1.4	Method of test	124

5.4.1.4.1	Initial conditions	124

5.4.1.4.2	Test sequence settings	125

5.4.1.4.3	Procedure	125

5.4.1.5	Acceptance criteria	125

5.4.1A	Unified Access Control – Access identity 0, no access identities indicated by USIM, Access Category 8	126

5.4.1A.1	Definition and applicability	126

5.4.1A.2	Conformance requirement	126

5.4.1A.3	Test purpose	127

5.4.1A.4	Method of test	127

5.4.1A.4.1	Initial conditions	127

5.4.1A.4.2	Test sequence settings	128

5.4.1A.4.3	Procedure	128

5.4.1A.5	Acceptance criteria	128

5.4.2	Unified Access Control – Access Identity 1 – MPS indicated by USIM	129

5.4.2.1	Definition and applicability	129

5.4.2.2	Conformance requirement	129

5.4.2.3	Test purpose	129

5.4.2.4	Method of test	130

5.4.2.4.1	Initial conditions	130

5.4.2.4.2	Test sequence settings	130

5.4.2.4.3	Procedure	131

5.4.2.5	Acceptance criteria	132

5.4.2A	Unified Access Control – Access Identity 1 – MPS indicated by USIM, Access Category 8	132

5.4.2A.1	Definition and applicability	132

5.4.2A.2	Conformance requirement	132

5.4.2A.3	Test purpose	133

5.4.2A.4	Method of test	133

5.4.2A.4.1	Initial conditions	133

5.4.2A.4.2	Test sequence settings	134

5.4.2A.4.3	Procedure	134

5.4.2A.5	Acceptance criteria	135

5.4.3	Unified Access Control – Access Identity 1 – no MPS indication by USIM and SUPI not changed	135

5.4.3.1	Definition and applicability	135

5.4.3.2	Conformance requirement	135

5.4.3.3	Test purpose	136

5.4.3.4	Method of test	137

5.4.3.4.1	Initial conditions	137

5.4.3.4.2	Test sequence settings	138

5.4.3.4.3	Procedure	138

5.4.3.5	Acceptance criteria	139

5.4.4	Unified Access Control – Access Identity 1 – no MPS indication by USIM and SUPI is changed	139

5.4.4.1	Definition and applicability	139

5.4.4.2	Conformance requirement	140

5.4.4.3	Test purpose	140

5.4.4.4	Method of test	141

5.4.4.4.1	Initial conditions	141

5.4.4.4.2	Test sequence settings	141

5.4.4.4.3	Procedure	142

5.4.4.5	Acceptance criteria	143

5.4.5	Unified Access Control – Access Identity 2 – MCS indicated by USIM	143

5.4.5.1	Definition and applicability	143

5.4.5.2	Conformance requirement	143

5.4.5.3	Test purpose	144

5.4.5.4	Method of test	144

5.4.5.4.1	Initial conditions	144

5.4.5.4.2	Test sequence settings	145

5.4.5.4.3	Procedure	145

5.4.5.5	Acceptance criteria	146

5.4.5A	Unified Access Control – Access Identity 2 – MCS indicated by USIM	146

5.4.5A.1	Definition and applicability	146

5.4.5A.2	Conformance requirement	147

5.4.5A.3	Test purpose	147

5.4.5A.4	Method of test	148

5.4.5A.4.1	Initial conditions	148

5.4.5A.4.2	Test sequence settings	148

5.4.5A.4.2	Procedure	149

5.4.5A.5	Acceptance criteria	149

5.4.6	Unified Access Control – Access Identity 2 – no MCS indication by USIM and SUPI is not changed	150

5.4.6.1	Definition and applicability	150

5.4.6.2	Conformance requirement	150

5.4.6.3	Test purpose	151

5.4.6.4	Method of test	151

5.4.6.4.1	Initial conditions	151

5.4.6.4.2	Test sequence settings	152

5.4.6.4.3	Procedure	152

5.4.6.4.3	Acceptance criteria	153

5.4.7	Unified Access Control – Access Identity 2 – no MCS indication by USIM and SUPI is changed	154

5.4.7.1	Definition and applicability	154

5.4.7.2	Conformance requirement	154

5.4.7.3	Test purpose	155

5.4.7.4	Method of test	155

5.4.7.4.1	Initial conditions	155

5.4.7.4.2	Test sequence settings	156

5.4.7.4.3	Procedure	156

5.4.7.5	Acceptance criteria	157

5.4.8	Unified Access Control – Access Identities 11 and 15 indicated by USIM	157

5.4.8.1	Definition and applicability	157

5.4.8.2	Conformance requirement	158

5.4.8.3	Test purpose	158

5.4.8.4	Method of test	158

5.4.8.4.1	Initial conditions	158

5.4.8.4.2	Test sequence settings	159

5.4.8.4.3	Procedure	160

5.4.8.5	Acceptance criteria	160

5.4.8A	Unified Access Control – Access Identities 11 and 15 indicated by USIM, Access Category 8	161

5.4.8A.1	Definition and applicability	161

5.4.8A.2	Conformance requirement	161

5.4.8A.3	Test purpose	162

5.4.8A.4	Method of test	162

5.4.8A.4.1	Initial conditions	162

5.4.8A.4.2	Test sequence settings	163

5.4.8A.4.3	Procedure	163

5.4.8A.5	Acceptance criteria	164

5.4.9	Unified Access Control – Access Identities 12, 13 and 14 indicated by USIM	164

5.4.9.1	Definition and applicability	164

5.4.9.2	Conformance requirement	164

5.4.9.3	Test purpose	165

5.4.9.4	Method of test	165

5.4.9.4.1	Initial conditions	165

5.4.9.4.2	Tables related to the test case	166

5.4.9.4.3	Procedure	167

5.4.9.5	Acceptance criteria	167

5.4.9A	Unified Access Control – Access Identities 12, 13 and 14 indicated by USIM, Access Category 8	168

5.4.9A.1	Definition and applicability	168

5.4.9A.2	Conformance requirement	168

5.4.9A.3	Test purpose	169

5.4.9A.4	Method of test	169

5.4.9A.4.1	Initial conditions	169

5.4.9A.4.2	Tables related to the test case	170

5.4.9A.4.3	Procedure	170

5.4.9A.5	Acceptance criteria	171

5.4.10	Unified Access Control – Operator-Defined Access Category	171

5.4.10.1	Definition and applicability	171

5.4.10.2	Conformance requirement	171

5.4.10.3	Test purpose	173

5.4.10.4	Method of test	173

5.4.10.4.1	Initial conditions	173

5.4.10.4.2	Tables related to the test case	174

5.4.10.4.3	Procedure	175

5.4.10.5	Acceptance criteria	176

5.4.11	Unified Access Control – Operator-Defined Access Categories, no change in SUPI	176

5.4.11.1	Definition and applicability	176

5.4.11.2	Conformance requirement	177

5.4.11.3	Test purpose	178

5.4.11.4	Method of test	178

5.4.11.4.1	Initial conditions	178

5.4.11.4.2	Tables related to the test case	180

5.4.11.4.3	Procedure	180

5.4.11.5	Acceptance criteria	181

5.4.12	Unified Access Control – Operator-Defined Access Categories, SUPI change	181

5.4.12.1	Definition and applicability	181

5.4.12.2	Conformance requirement	182

5.4.12.3	Test purpose	183

5.4.12.4	Method of test	184

5.4.12.4.1	Initial conditions	184

5.4.12.4.2	Tables related to the test case	185

5.4.12.4.3	Procedure	186

5.4.12.5	Acceptance criteria	187

5.5	Handling of operator controlled features	187

5.5.1	Display of registered 5G PLMN name from USIM	187

5.5.1.1	Definition and applicability	187

5.5.1.2	Conformance requirement	187

5.5.1.3	Test purpose	187

5.5.1.4	Method of tests	187

5.5.1.4.1	Initial conditions	187

5.5.1.4.2	Procedure	190

5.5.1.4.3	Acceptance criteria	190

5.5.2	Display of registered 5G PLMN name from ME	191

5.5.2.1	Definition and applicability	191

5.5.2.2	Conformance requirement	191

5.5.2.3	Test purpose	191

5.5.2.4	Method of tests	191

5.5.2.4.1	Initial conditions	191

5.5.2.4.2	Procedure	193

5.5.1.4.3	Acceptance criteria	194

5.6.1.1	Definition and applicability	194

5.6.1.2	Conformance requirement	194

5.6.1.3	Test purpose	195

5.6.1.4	Method of test	195

5.6.1.4.1	Initial conditions	195

5.6.1.4.2	Procedure	196

5.6.1.5	Acceptance criteria	197

5.6.2.1	Definition and applicability	197

5.6.2.2	Conformance requirement	197

5.6.2.3	Test purpose	198

5.6.2.4	Method of test	198

5.6.2.4.1	Initial conditions	198

5.6.2.4.2	Procedure	200

5.6.2.5	Acceptance criteria	200

5.6.3.1	Definition and applicability	200

5.6.3.2	Conformance requirement	201

5.6.3.3	Test purpose	201

5.6.3.4	Method of test	202

5.6.3.4.1	Initial conditions	202

5.6.3.4.2	Procedure	203

5.6.3.5	Acceptance criteria	203

5.6.4	UE identification after SUPI is changed	204

5.6.4.1	Definition and applicability	204

5.6.4.2	Conformance requirement	204

5.6.4.3	Test purpose	204

5.6.4.4	Method of test	205

5.6.4.4.1	Initial conditions	205

5.6.4.4.2	Procedure	205

5.6.4.5	Acceptance criteria	206

5.6.5	UE identification by SUCI during initial registration – SUCI calculation by ME using profile A	206

5.6.5.1	Definition and applicability	206

5.6.5.2	Conformance requirement	206

5.6.5.3	Test purpose	207

5.6.5.4	Method of test	207

5.6.5.4.1	Initial conditions	207

5.6.5.4.2	Procedure	208

5.6.5.5	Acceptance criteria	209

5.6.6	UE identification by SUCI during initial registration – SUCI calculation by ME using profile A	209

5.6.6.1	Definition and applicability	209

5.6.6.2	Conformance requirement	209

5.6.6.3	Test purpose	210

5.6.6.4	Method of test	210

5.6.6.4.1	Initial conditions	210

5.6.6.4.2	Procedure	211

5.6.5.5	Acceptance criteria	211

6	Security related Tests	212

6.1	PIN handling	212

6.1.1	Entry of PIN	212

6.1.1.1	Definition and applicability	212

6.1.1.2	Conformance requirement	212

6.1.1.3	Test purpose	212

6.1.1.4	Method of test	212

6.1.1.4.1	Initial conditions	212

6.1.1.4.2	Procedure	212

6.1.1.5	Acceptance criteria	213

6.1.2	Change of PIN	213

6.1.2.1	Definition and applicability	213

6.1.2.2	Conformance requirement	213

6.1.2.3	Test purpose	213

6.1.2.4	Method of test	213

6.1.2.4.1	Initial conditions	213

6.1.2.4.2	Procedure	213

6.1.2.5	Acceptance criteria	214

6.1.3	Unblock PIN	214

6.1.3.1	Definition and applicability	214

6.1.3.2	Conformance requirement	214

6.1.3.3	Test purpose	215

6.1.3.4	Method of test	215

6.1.3.4.1	Initial conditions	215

6.1.3.4.2	Procedure	215

6.1.3.4.2	Procedure	215

6.1.3.5	Acceptance criteria	216

6.1.4	Entry of PIN2	217

6.1.4.1	Definition and applicability	217

6.1.4.2	Conformance requirement	217

6.1.4.3	Test purpose	217

6.1.4.4	Method of test	217

6.1.4.4.1	Initial conditions	217

6.1.4.4.2	Procedure	217

6.1.4.5	Acceptance criteria	218

6.1.5	Change of PIN2	218

6.1.5.1	Definition and applicability	218

6.1.5.2	Conformance requirement	218

6.1.5.3	Test purpose	218

6.1.5.4	Method of test	218

6.1.5.4.1	Initial conditions	218

6.1.5.4.2	Procedure	218

6.1.5.5	Acceptance criteria	219

6.1.6	Unblock PIN2	219

6.1.6.1	Definition and applicability	219

6.1.6.2	Conformance requirement	219

6.1.6.3	Test purpose	220

6.1.6.4	Method of test	220

6.1.6.4.1	Initial conditions	220

6.1.6.4.2	Procedure	220

6.1.6.5	Acceptance criteria	221

6.1.7	Replacement of PIN	222

6.1.7.1	Definition and applicability	222

6.1.7.2	Conformance requirement	222

6.1.7.3	Test purpose	222

6.1.7.4	Method of test	222

6.1.7.4.1	Initial conditions	222

6.1.7.4.2	Procedure	222

6.1.7.5	Acceptance criteria	223

6.1.8	Change of Universal PIN	223

6.1.8.1	Definition and applicability	223

6.1.8.2	Conformance requirement	224

6.1.8.3	Test purpose	224

6.1.8.4	Method of test	224

6.1.8.4.1	Initial conditions	224

6.1.8.4.2	Procedure	224

6.1.8.5	Acceptance criteria	225

6.1.9	Unblock Universal PIN	225

6.1.9.1	Definition and applicability	225

6.1.9.2	Conformance requirement	225

6.1.9.3	Test purpose	225

6.1.9.4	Method of test	225

6.1.9.4.1	Initial conditions	225

6.1.9.4.2	Procedure	225

6.1.9.5	Acceptance criteria	226

6.1.10	Entry of PIN on multi-verification capable UICCs	226

6.1.10.1	Definition and applicability	226

6.1.10.2	Conformance requirement	226

6.1.10.4	Method of test	227

6.1.10.4.1	Initial conditions	227

6.1.10.4.2	Procedure	228

6.1.10.5	Acceptance criteria	228

6.1.11	Change of PIN on multi-verification capable UICCs	228

6.1.11.1	Definition and applicability	228

6.1.11.2	Conformance requirement	228

6.1.11.3	Test purpose	228

6.1.11.4	Method of test	229

6.1.11.4.1	Initial conditions	229

6.1.11.4.2	Procedure	229

6.1.11.5	Acceptance criteria	230

6.1.12	Unblock PIN on multi-verification capable UICCs	230

6.1.12.1	Definition and applicability	230

6.1.12.2	Conformance requirement	230

6.1.12.3	Test purpose	231

6.1.12.4	Method of test	231

6.1.12.4.1	Initial conditions	231

6.1.12.4.2	Procedure	232

6.1.12.5	Acceptance criteria	233

6.1.13	Entry of PIN2 on multi-verification capable UICCs	233

6.1.13.1	Definition and applicability	233

6.1.13.2	Conformance requirement	233

6.1.13.3	Test purpose	233

6.1.13.4	Method of test	234

6.1.13.4.1	Initial conditions	234

6.1.13.4.2	Procedure	235

6.1.13.5	Acceptance criteria	235

6.1.14	Change of PIN2 on multi-verification capable UICCs	235

6.1.14.1	Definition and applicability	235

6.1.14.2	Conformance requirement	235

6.1.14.3	Test purpose	235

6.1.14.4	Method of test	236

6.1.14.4.1	Initial conditions	236

6.1.14.4.2	Procedure	237

6.1.14.5	Acceptance criteria	237

6.1.15	Unblock PIN2 on multi-verification capable UICCs	237

6.1.15.1	Definition and applicability	237

6.1.15.2	Conformance requirement	237

6.1.15.3	Test purpose	238

6.1.15.4	Method of test	238

6.1.15.4.1	Initial conditions	238

6.1.15.4.2	Procedure	239

6.1.15.5	Acceptance criteria	240

6.1.16	Replacement of PIN with key reference "07"	241

6.1.16.1	Definition and applicability	241

6.1.16.2	Conformance requirement	241

6.1.16.3	Test purpose	241

6.1.16.4	Method of test	241

6.1.16.4.1	Initial conditions	241

6.1.16.4.2	Procedure	242

6.1.16.5	Acceptance criteria	243

6.2	Fixed Dialling Numbers (FDN) handling	243

6.2.1	ME and USIM with FDN enabled, EFADN readable and updateable	243

6.2.2	ME and USIM with FDN disabled	243

6.2.3	Enabling, disabling and updating of FDN	243

6.2.4	ME and USIM with FDN enabled, EFADN readable and updateable (Rel-4 and onwards)	243

6.3	Void	244

6.4	Advice of charge (AoC) handling	244

6.4.1	AoC not supported by USIM	244

6.4.2	Maximum frequency of ACM updating	244

6.4.3	Call terminated when ACM greater than ACMmax	244

6.4.4	Response codes of increase command of ACM	244

7	PLMN related tests	244

7.1	FPLMN handling	244

7.1.1	Adding FPLMN to the Forbidden PLMN list	244

7.1.2	UE updating forbidden PLMNs	244

7.1.3	UE deleting forbidden PLMNs	244

7.1.4	Adding FPLMN to the forbidden PLMN list when accessing E-UTRAN	244

7.1.4.1	Definition and applicability	244

7.1.4.2	Conformance requirement	244

7.1.4.3	Test purpose	245

7.1.4.4	Method of test	245

7.1.4.4.1	Initial conditions	245

7.1.4.4.2	Procedure	245

7.1.4.4	Acceptance criteria	246

7.1.5	UE updating forbidden PLMNs when accessing E-UTRAN	248

7.1.5.1	Definition and applicability	248

7.1.5.2	Conformance requirement	248

7.1.5.3	Test purpose	248

7.1.5.4	Method of test	248

7.1.5.4.1	Initial conditions	248

7.1.5.4.2	Procedure	248

7.1.5.5	Acceptance criteria	248

7.1.6	UE deleting forbidden PLMNs when accessing E-UTRAN	249

7.1.6.1	Definition and applicability	249

7.1.6.2	Conformance requirement	249

7.1.6.3	Test purpose	250

7.1.6.4	Method of test	250

7.1.6.4.1	Initial conditions	250

7.1.6.4.2	Procedure	250

7.1.6.5	Acceptance criteria	250

7.1.7	Updating the Forbidden PLMN list after receiving non-integrity protected reject message – UTRAN	251

7.1.8	Updating the Forbidden PLMN list after receiving non-integrity protected reject message – E-UTRAN	251

7.1.8.1	Definition and applicability	251

7.1.8.2	Conformance requirement	251

7.1.8.3	Test purpose	252

7.1.8.4	Method of test	252

7.1.8.4.1	Initial conditions	252

7.1.8.4.2	Procedure	252

7.1.8.5	Acceptance criteria	253

7.2	User controlled PLMN selector handling	253

7.2.1	UE updating the User controlled PLMN selector list	253

7.2.1.1	Definition and applicability	253

7.2.1.2	Conformance requirement	254

7.2.1.3	Test purpose	254

7.2.1.4	Method of test	254

7.2.1.4.1	Initial conditions	254

7.2.1.4.2	Procedure	254

7.2.1.5	Acceptance criteria	254

7.2.2	UE recognizing the priority order of the User controlled PLMN selector list with the same access Technology	255

7.2.3	UE recognizing the priority order of the User controlled PLMN selector list using an ACT Preference	255

7.2.4	Void	255

7.2.5	UE updating the User controlled PLMN selector list for E-UTRAN	255

7.2.5.1	Definition and applicability	255

7.2.5.2	Conformance requirement	255

7.2.5.3	Test purpose	255

7.2.5.4	Method of test	256

7.2.5.4.1	Initial conditions	256

7.2.5.4.2	Procedure	256

7.2.5.5	Acceptance criteria	256

7.2.6	UE recognizing the priority order of the User controlled PLMN selector list using an ACT preference - UTRAN/E-UTRAN	257

7.2.7	UE recognizing the priority order of the User controlled PLMN selector list using an ACT preference - GSM/E-UTRAN	257

7.2.8	UE recognising the priority order of the User controlled PLMN selector list with the same access technology – E-UTRAN in NB-S1 mode	257

7.2.8.1	Definition and applicability	257

7.2.8.2	Conformance requirement	257

7.2.8.3	Test purpose	257

7.2.8.4	Method of test	257

7.2.8.4.1	Initial conditions	257

7.2.8.4.2	Procedure	258

7.2.8.5	Acceptance criteria	259

7.2.9	UE recognising the priority order of the User controlled PLMN selector list using the ACT preference – E-UTRAN in WB-S1/E-UTRAN in NB-S1	259

7.2.9.1	Definition and applicability	259

7.2.9.2	Conformance requirement	260

7.2.9.3	Test purpose	260

7.2.9.4	Method of test	260

7.2.9.4.1	Initial conditions	260

7.2.9.4.2	Procedure	261

7.2.9.5	Acceptance criteria	261

7.3	Operator controlled PLMN selector handling	262

7.3.1	UE recognizing the priority order of the Operator controlled PLMN selector list	262

7.3.2	UE recognizing the priority order of the User controlled PLMN selector over the Operator controlled PLMN selector list	262

7.3.3	UE recognizing the priority order of the Operator controlled PLMN selector list when accessing E-UTRAN	262

7.3.3.1	Definition and applicability	262

7.3.3.2	Conformance requirement	262

7.3.3.3	Test purpose	262

7.3.3.4	Method of test	262

7.3.3.4.1	Initial conditions	262

7.3.3.4.2	Procedure	264

7.3.3.5	Acceptance criteria	264

7.3.4	UE recognizing the priority order of the User controlled PLMN selector over the Operator controlled PLMN selector list – E-UTRAN	265

7.3.4.1	Definition and applicability	265

7.3.4.2	Conformance requirement	265

7.3.4.3	Test purpose	265

7.3.4.4	Method of test	265

7.3.4.4.1	Initial conditions	265

7.3.4.4.2	Procedure	266

7.3.4.5	Acceptance criteria	266

7.3.5	UE recognising the priority order of the Operator controlled PLMN selector list when accessing E-UTRAN in NB-S1 mode	267

7.3.5.1	Definition and applicability	267

7.3.5.2	Conformance requirement	267

7.3.5.3	Test purpose	267

7.3.5.4	Method of test	267

7.3.5.4.1	Initial conditions	267

7.3.5.4.2	Procedure	268

7.3.5.5	Acceptance criteria	269

7.3.6	UE recognising the priority order of the User controlled PLMN selector over the Operator controlled PLMN selector list – E-UTRAN in NB-S1 mode	269

7.3.6.1	Definition and applicability	269

7.3.6.2	Conformance requirement	269

7.3.6.3	Test purpose	270

7.3.6.4	Method of test	270

7.3.6.4.1	Initial conditions	270

7.3.6.4.2	Procedure	272

7.3.6.5	Acceptance criteria	272

7.3.7	UE recognising the priority order of the Operator controlled PLMN selector list using the ACT preference – E-UTRAN in NB-S1/ E-UTRAN in WB-S1 mode	272

7.3.7.1	Definition and applicability	272

7.3.7.2	Conformance requirement	273

7.3.7.3	Test purpose	273

7.3.7.4	Method of test	273

7.3.7.4.1	Initial conditions	273

7.3.7.4.2	Procedure	274

7.3.7.5	Acceptance criteria	274

7.3.8	UE recognising the priority order of the Operator controlled PLMN selector list using the ACT preference – E-UTRAN in NB-S1 mode/ GSM	275

7.3.9	UE recognising the priority order of the Operator controlled PLMN selector list using the ACT preference – E-UTRAN in WB-S1 mode/GSM	275

7.4	Higher priority PLMN search handling	275

7.4.1	UE recognizing the search period of the Higher priority PLMN	275

7.4.2	GSM/UMTS dual mode UEs recognizing the search period of the Higher priority PLMN	275

7.4.3	UE recognizing the search period of the Higher priority PLMN – E-UTRAN	275

7.4.3.1	Definition and applicability	275

7.3.7.2	Conformance requirement	275

7.4.3.3	Test purpose	276

7.4.3.4	Method of test	276

7.4.3.4.1	Initial conditions	276

7.4.3.4.2	Procedure	276

7.4.3.5	Acceptance criteria	277

7.4.4	E-UTRAN/EPC capable UEs recognizing the search period of the Higher priority PLMN – GSM/E-UTRAN	278

7.4.5	E-UTRAN/EPC capable UEs recognizing the search period of the Higher priority PLMN – UTRAN/E-UTRAN	278

7.4.6	UE recognising the search period of the higher priority PLMN – Satellite NG-RAN	278

7.4.7	UE recognising the search period of the higher priority PLMN – NG-RAN	278

7.4.7.1	Definition and applicability	278

7.4.7.2	Conformance requirement	278

7.4.7.3	Test purpose	279

7.4.7.4	Method of test	279

7.4.7.4.1	Initial conditions	279

7.4.7.4.2	Procedure	280

7.4.7.5	Acceptance criteria	281

7.5	Void	282

8	Subscription independent tests	282

8.1	Phone book procedures	282

8.1.1	Recognition of a previously changed phonebook	282

8.1.2	Update of the Phonebook Synchronization Counter (PSC)	282

8.1.2.1	Definition and applicability	282

8.1.2.2	Conformance requirement	282

8.1.2.3	Test purpose	282

8.1.2.4	Method of test	282

8.1.2.4.1	Initial conditions	282

8.1.2.4.2	Procedure	283

8.1.2.5	Acceptance criteria	284

8.1.3	Handling of BCD number/ SSC content extension	284

8.1.3.1	Definition and applicability	284

8.1.3.2	Conformance requirement	284

8.1.3.3	Test purpose	284

8.1.3.4	Method of test	285

8.1.3.4.1	Initial conditions	285

8.1.3.4.2	Procedure	287

8.1.3.5	Acceptance criteria	289

8.1.4	Phonebook selection	289

8.1.4.1	Definition and applicability	289

8.1.4.2	Conformance requirement	289

8.1.4.3	Test purpose	290

8.1.4.4	Method of test	290

8.1.4.4.1	Initial conditions	290

8.1.4.4.2	Procedure	293

8.1.4.5	Acceptance criteria	294

8.1.5	Local Phonebook handling	295

8.1.5.1	Definition and applicability	295

8.1.5.2	Conformance requirement	295

8.1.5.3	Test purpose	295

8.1.5.4	Method of test	295

8.1.5.4.1	Initial conditions	295

8.1.5.4.2	Procedure	296

8.1.5.5	Acceptance criteria	297

8.2	Short message handling report	298

8.2.1	Correct storage of a SM on the USIM	298

8.2.2	Correct reading of a SM on the USIM	298

8.2.3	SM memory capacity exceeded handling	298

8.2.4A	Correct storage of an SM on the UICC	298

8.2.4B	Correct storage of an SM on the UICC	298

8.2.5	Correct reading of a SM on the USIM if USIM and ISIM are present	298

8.2.6	Correct reading of a SM on the ISIM if USIM and ISIM are present	298

8.2.7	Correct storage of an SM on the UICC	298

8.3	MMS related tests	299

8.3.0	General condition	299

8.3.1	UE recognizing the priority order of MMS Issuer Connectivity Parameters	299

8.3.2	UE recognizing the priority order of MMS User Connectivity Parameters	299

8.3.3	UE recognizing the priority order of MMS Issuer Connectivity Parameters over the MMS User Connectivity Parameters	299

8.3.4	Usage of MMS notification	299

8.4	UICC presence detection	299

8.5	UICC presence detection when connected to E-UTRAN/EPC	299

9	USIM service handling	299

9.1	Access Point Name Control List handling	299

9.1.1	Access Point Name Control List handling for UEs supporting ACL	299

9.1.2	Network provided APN handling for UEs supporting ACL	299

9.1.3	Access Point Name Control List handling for UEs not supporting ACL	300

9.1.4	Access Point Name Control List handling for UEs supporting ACL connected to E-UTRAN/EPC	300

9.1.4.1	Definition and applicability	300

9.1.4.2	Conformance requirement	300

9.1.4.3	Test purpose	300

9.1.4.4	Method of test	300

9.1.4.4.1	Initial conditions	300

9.1.4.4.2	Procedure	301

9.1.4.5	Acceptance criteria	302

9.2	Service Dialling Numbers handling	302

9.2.1	Service Dialling Numbers handling	302

10	CSG list handling	302

10.1	CSG list handling for E-UTRA	302

10.1.1	Automatic CSG selection in E-UTRA with CSG list on USIM, success	302

10.1.1.1	Definition and applicability	302

10.1.1.2	Conformance requirement	302

10.1.1.3	Test purpose	303

10.1.1.4	Method of test	303

10.1.1.4.1	Initial conditions	303

10.1.1.4.2	Procedure	303

10.1.1.5	Acceptance criteria	304

10.1.2.1	Definition and applicability	304

10.1.2.2	Conformance requirement	304

10.1.2.3	Test purpose	305

10.1.2.4	Method of test	305

10.1.2.4.1	Initial conditions	305

10.1.2.4.2	Procedure	305

10.1.2.5	Acceptance criteria	305

10.1.3	Manual CSG selection in E-UTRA with CSG list on USIM, success	306

10.1.3.1	Definition and applicability	306

10.1.3.2	Conformance requirement	306

10.1.3.3	Test purpose	307

10.1.3.4	Method of test	307

10.1.3.4.1	Initial conditions	307

10.1.3.4.2	Procedure	307

10.1.3.5	Acceptance criteria	308

10.1.4	Manual CSG selection in E-UTRA with CSG list on USIM, rejected	309

10.1.4.1	Definition and applicability	309

10.1.4.2	Conformance requirement	309

10.1.4.3	Test purpose	309

10.1.4.4	Method of test	310

10.1.4.4.1	Initial conditions	310

10.1.4.4.2	Procedure	310

10.1.4.5	Acceptance criteria	310

10.1.5	CSG selection in E-UTRA with no CSG list on USIM, no IMSI change	311

10.1.5.1	Definition and applicability	311

10.1.5.2	Conformance requirement	311

10.1.5.3	Test purpose	312

10.1.5.4	Method of test	312

10.1.5.4.1	Initial conditions	312

10.1.5.4.2	Procedure	312

10.1.5.5	Acceptance criteria	313

10.1.6	CSG selection in E-UTRA with no CSG list on USIM, with IMSI change	314

10.1.6.1	Definition and applicability	314

10.1.6.2	Conformance requirement	314

10.1.6.3	Test purpose	314

10.1.6.4	Method of test	314

10.1.6.4.1	Initial conditions	314

10.1.6.4.2	Procedure	315

10.1.6.5	Acceptance criteria	316

10.1.7	Manual CSG selection without display restrictions in E-UTRA with ACSG list and OCSG list on USIM	316

10.1.7.1	Definition and applicability	316

10.1.7.2	Conformance requirement	316

10.1.7.3	Test purpose	317

10.1.7.4	Method of test	317

10.1.7.4.1	Initial conditions	317

10.1.7.4.2	Procedure	317

10.1.7.5	Acceptance criteria	318

10.1.8	Manual CSG selection with display restrictions in E-UTRA with ACSG list and OCSG list on USIM	319

10.1.8.1	Definition and applicability	319

10.1.8.2	Conformance requirement	319

10.1.8.3	Test purpose	320

10.1.8.4	Method of test	320

10.1.8.4.1	Initial conditions	320

10.1.8.4.2	Procedure	320

10.1.8.5	Acceptance criteria	321

10.2	CSG list handling for UTRA	322

10.2.1	Manual CSG selection without display restrictions in UTRA with ACSG list and OCSG list on USIM	322

10.2.2	Manual CSG selection with display restrictions in UTRA with ACSG list and OCSG list on USIM	323

10.2.3	Manual CSG selection in UTRA with CSG list on USIM, success	323

11	NAS security context parameter handling	323

11.1	NAS security context parameter handling when service "EMM Information" is available	323

11.1.1	Definition and applicability	323

11.1.2	Conformance requirement	323

11.1.3	Test purpose	323

11.1.4	Method of test	323

11.1.4.1	Initial conditions	323

11.1.4.2	Procedure	324

11.1.4.5	Acceptance criteria	324

11.2	NAS security context parameter handling when service "EMM Information" is not available, no IMSI change	325

11.2.1	Definition and applicability	325

11.2.2	Conformance requirement	325

11.2.3	Test purpose	326

11.2.4	Method of test	326

11.2.4.1	Initial conditions	326

11.2.4.2	Procedure	326

11.2.5	Acceptance criteria	327

11.3	NAS security context parameter handling when service "EMM Information" is not available, IMSI Changed	327

11.3.1	Definition and applicability	327

11.3.2	Conformance requirement	327

11.3.3	Test purpose	328

11.3.4	Method of test	328

11.3.4.1	Initial conditions	328

11.3.4.2	Procedure	328

11.3.5	Acceptance criteria	329

11.4	EPS NAS Security Context Storage	329

11.4.1	Definition and applicability	329

11.4.2	Conformance requirement	329

11.4.3	Test purpose	330

11.4.4	Method of test	330

11.4.4.1	Initial conditions	330

11.4.4.2	Procedure	330

11.4.5	Acceptance criteria	331

12	Non Access Stratum (NAS) Configuration parameter handling	331

12.1	EFNASCONFIG - NAS signalling priority handling	331

12.2	EFNASCONFIG - NMO I Network Mode of Operation I handling	331

12.3	EFNASCONFIG - Attach with IMSI handling	331

12.4	EFNASCONFIG - Verifying Minimum Periodic Search Timer	331

12.5	EFNASCONFIG - Extended access barring handling	332

12.6	EFNASCONFIG - Verifying Timer T3245 Behaviour	332

12.7	EFNASCONFIG - Override NAS signalling low priority	332

12.8	EFNASCONFIG - Override Extended access barring	332

12.9	EFNASCONFIG - Fast First Higher Priority PLMN Search	332

12.10	EFNASCONFIG - E-UTRA Disabling Allowed for EMM cause #15	332

12.11	EFNASCONFIG - SM_RetryWaitTime	332

12.12	EFNASCONFIG - SM_RetryAtRATChange	332

13	UICC interface during PSM	332

13.1	UICC interface in PSM handling for E-UTRAN – No UICC deactivation in PSM	332

13.1.1	Definition and applicability	332

13.1.2	Conformance requirement	333

13.1.3	Test purpose	333

13.1.4	Method of test	333

13.1.4.1	Initial conditions	333

13.1.4.2	Procedure	333

13.1.5	Acceptance criteria	334

13.2	UICC interface in PSM handling for E-UTRAN – PSM not accepted by E-USS/NB-SS	334

13.2.1	Definition and applicability	334

13.2.2	Conformance requirement	334

13.2.3	Test purpose	334

13.2.4	Method of test	334

13.2.4.1	Initial conditions	334

13.2.4.2	Procedure	335

13.2.5	Acceptance criteria	335

13.3	UICC interface in PSM handling for E-UTRAN – UICC deactivation in PSM	335

13.4	UICC interface in PSM for E-UTRAN – SUSPEND UICC	336

13.4.1	Definition and applicability	336

13.4.2	Conformance requirement	336

13.4.3	Test purpose	336

13.4.4	Method of test	336

13.4.4.1	Initial conditions	336

13.4.4.2	Procedure	337

13.4.5	Acceptance criteria	337

14	UICC interface during eDRX	337

14.1	UICC interface during eDRX for E-UTRAN – eDRX is not supported by the UICC	337

14.1.1	Definition and applicability	337

14.1.2	Conformance requirement	338

14.1.3	Test purpose	338

14.1.4	Method of test	338

14.1.4.1	Initial conditions	338

14.1.4.2	Procedure	338

14.1.5	Acceptance criteria	339

14.2	UICC interface during eDRX for E-UTRAN – eDRX is not accepted by E-USS/NB-SS	339

14.2.1	Definition and applicability	339

14.2.2	Conformance requirement	339

14.2.3	Test purpose	340

14.2.4	Method of test	340

14.2.4.1	Initial conditions	340

14.2.4.2	Procedure	340

14.2.5	Acceptance criteria	341

14.3	UICC interface during eDRX for E-UTRAN – UICC deactivation during eDRX	341

14.4	UICC interface during eDRX for E-UTRAN– SUSPEND UICC	341

15	Authentication procedure and NAS security context handling for 5G	341

15.1	Authentication procedure for EAP-AKA'	341

15.1.1	Authentication procedure for EAP-AKA' - Authentication is successful	341

15.1.1.1	Definition and applicability	341

15.1.1.2	Conformance requirement	342

15.1.1.3	Test purpose	342

15.1.1.4	Method of test	343

15.1.1.4.1	Initial conditions	343

15.1.1.4.2	Procedure	343

15.1.1.5	Acceptance criteria	344

15.1.1.5	Acceptance criteria	345

15.1A.1	Authentication procedure for EAP-AKA' - Authentication is successful	345

15.1A.1.1	Definition and applicability	345

15.1A.1.2	Conformance requirement	345

15.1A.1.3	Test purpose	346

15.1A.1.4	Method of test	346

15.1A.1.4.1	Initial conditions	346

15.1A.1.4.2	Procedure	346

15.1A.1.5	Acceptance criteria	347

15.1.2	Authentication procedure for EAP-AKA' – Authentication is successful - GSM UICC	348

15.1.2.1	Definition and applicability	348

15.1.2.2	Conformance requirement	348

15.1.2.3	Test purpose	349

15.1.2.4	Method of test	349

15.1.2.4.1	Initial conditions	349

15.1.2.4.2	Procedure	349

15.1.2.5	Acceptance criteria	350

15.1.3	Authentication procedure for EAP-AKA' – AUTN fails on the USIM	350

15.1.3.1	Definition and applicability	350

15.1.3.2	Conformance requirement	351

15.1.3.3	Test purpose	351

15.1.3.4	Method of test	351

15.1.3.4.1	Initial conditions	351

15.1.3.4.2	Procedure	351

15.1.3.5	Acceptance criteria	352

15.1.4	Authentication procedure for EAP-AKA' - after SUPI is changed	352

15.1.4.1	Definition and applicability	352

15.1.4.2	Conformance requirement	353

15.1.4.3	Test purpose	353

15.1.4.4	Method of test	353

15.1.4.4.2	Procedure	353

15.1.4.5	Acceptance criteria	355

15.2	Authentication procedure for 5G AKA	355

15.2.1	Authentication procedure for 5G AKA - Authentication is successful	355

15.2.1.4	Method of test	356

15.2.1.4.1	Initial conditions	356

15.2.1.5	Acceptance criteria	357

15.2A.1	Authentication procedure for 5G AKA - Authentication is successful	358

15.2A.1.4	Method of test	359

15.2A.1.4.1	Initial conditions	359

15.2A.1.5	Acceptance criteria	360

15.2.2	Authentication procedure for 5G AKA – Authentication is successful - GSM UICC	361

15.2.2.1	Definition and applicability	361

15.2.2.2	Conformance requirement	361

15.2.2.3	Test purpose	362

15.2.2.4	Method of test	362

15.2.2.4.1	Initial conditions	362

15.2.2.4.2	Procedure	362

15.2.2.5	Acceptance criteria	363

15.2A.2	Authentication procedure for 5G AKA - Authentication is successful	363

15.2A.2.1	Definition and applicability	363

15.2A.2.2	Conformance requirement	364

15.2A.2.3	Test purpose	364

15.2A.2.4	Method of test	364

15.2A.2.4.1	Initial conditions	364

15.2A.2.4.2	Procedure	364

15.2A.2.5	Acceptance criteria	365

15.2.3	Authentication procedure 5G AKA – AUTN fails on the USIM	365

15.2.3.1	Definition and applicability	365

15.2.3.2	Conformance requirement	366

15.2.3.3	Test purpose	366

15.2.3.4	Method of test	366

15.2.3.4.1	Initial conditions	366

15.2.3.4.2	Procedure	366

15.2.3.5	Acceptance criteria	367

15.2.4	Authentication procedure for 5G AKA - after SUPI is changed	368

15.2.4.1	Definition and applicability	368

15.2.4.2	Conformance requirement	368

15.2.4.3	Test purpose	368

15.2.4.4	Method of test	368

15.2.4.4.1	Initial conditions	368

15.2.4.4.2	Procedure	369

15.2.4.5	Acceptance criteria	370

15.2.5.2	Conformance requirement	371

15.2.5.4	Method of test	372

15.2.5.4.1	Initial conditions	372

15.2.5.4.2	Procedure	372

15.2.5.5	Acceptance criteria	373

16	UE Route Selection Policy (URSP) procedure	376

16.1	Pre-configured URSP rules	376

16.1.1	Support for URSP by USIM	376

16.1.1.1	Definition and applicability	376

16.1.1.2	Conformance requirement	376

16.1.1.3	Test purpose	376

16.1.1.4	Method of test	377

16.1.1.4.1	Initial conditions	377

16.1.1.4.2	Procedure	378

16.1.1.5	Acceptance criteria	378

16.1.2	Support for URSP by ME	379

16.1.2.1	Definition and applicability	379

16.1.2.2	Conformance requirement	379

16.1.2.3	Test purpose	379

16.1.2.4	Method of test	379

16.1.2.4.1	Initial conditions	379

16.1.2.4.2	Procedure	380

16.1.2.5	Acceptance criteria	380

16.1.3	Support of Signalled URSP	381

16.1.3.1	Definition and applicability	381

16.1.3.2	Conformance requirement	381

16.1.3.3	Test purpose	381

16.1.3.4	Method of test	381

16.1.3.4.1	Initial conditions	381

16.1.3.4.2	Procedure	382

16.1.3.5	Acceptance criteria	383

17	CAG list handling	383

17.1	CAG list handling for 5G	383

17.1.1	Automatic CAG selection with preconfigured CAG list on USIM	383

17.1.1.1	Definition and applicability	383

17.1.1.2	Conformance requirement	383

17.1.1.3	Test purpose	384

17.1.1.4	Method of tests	384

17.1.1.4.1	Initial conditions	384

17.1.1.4.2	Procedure	384

17.1.1.5	Acceptance criteria	385

18	NSSAA procedure with SSIM	385

18.1	SSIM management procedures	385

18.1.1	SSIM identification, S-NSSAI list request	385

18.1.1.1	Definition and applicability	385

18.1.1.2	Conformance requirement	385

18.1.1.3	Test purpose	386

18.1.1.4	Method of tests	386

18.1.1.4.1	Initial conditions	386

18.1.1.4.2	Procedure	387

18.1.1.4.2	Acceptance criteria	388

18.2	SSIM commands	388

18.2.1	AUTHENTICATE command, Identity EAP-Request/EAP-Response messages	388

18.2.1.1	Definition and applicability	388

18.2.1.2	Conformance requirement	389

18.2.1.3	Test purpose	389

18.2.1.4	Method of tests	389

18.2.1.4.1	Initial conditions	389

18.2.1.4.2	Procedure	389

18.2.1.4.3	Acceptance criteria	390

18.2.2	AUTHENTICATE command, other EAP-Request/EAP-Response messages	390

18.2.2.1	Definition and applicability	390

18.2.2.2	Conformance requirement	391

18.2.2.3	Test purpose	391

18.2.2.4	Method of tests	391

18.2.2.4.1	Initial conditions	391

18.2.2.4.2	Procedure	391

18.2.2.4.3	Acceptance criteria	391

18.2.3	AUTHENTICATE command, EAP-Success/EAP-Failure messages	392

18.2.3.1	Definition and applicability	392

18.2.3.2	Conformance requirement	392

18.2.3.3	Test purpose	392

18.2.3.4	Method of tests	392

18.2.3.4.1	Initial conditions	392

18.2.3.4.2	Procedure	392

18.2.3.4.3	Acceptance criteria	393

Annex A (informative):	Suggested requirements lists for Test Applet functionality	393

A.1	General requirements	393

A.2	Test instruction or input data requirements	394

A.2.1	Test instruction or input data requirements list-1	394

A.2.2	Test instruction or input data requirements list-2	394

A.3	APDU content verification requirements	394

A.3.1	APDU content verification requirements list-1	394

A.3.2	APDU content verification requirements list-2	395

Annex B (informative):	Examples of Test EFs in the Test-nrUICC configuration	395

B.1	Test EF structure - 1	395

B.1.1	EFSTK (SIMToolkit data)	396

B.1.2	EFSETSTK (Set SIM Toolkit)	396

B.1.3	EFCC (CALL CONTROL)	397

B.1.4	EFEVENTLIST (EVENT LIST)	397

B.2	Test EF structure - 2	397

B.2.1	EFTC_IN (Test Case Input)	398

B.2.2	EFTC_OUT (Test Case Output)	399

B.2.3	EFEF_RESET (EF_RESET)	399

Annex C (informative):	Add a title	401

C.1	5G-NR generic procedure for 5G Registration on 3GPP access with IMS service	401

C.1.1	'voice centric' Procedure	401

Annex D (informative):	APDU logging in seamless testing	402

D.1	Example of SIM APDU logging via AT interface	402

Annex E (informative):	Change history	403

# Foreword

This Technical Specification| has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

The present document defines application behavioural tests for a UE with a non-removable UICC/USIM where no access to the physical UICC-Terminal interface can be granted when interacting with a 3GPP network.

The aim of the present document is to ensure the correct behaviour of a UE interfacing with a 3GPP network whilst it is proved that card specific data and functionality is used wherever indicated, similar to application tests defined in TS 31.121 [2] but without direct access to the UICC-Terminal interface.

The present document does not define any aspects related to the administrative management phase of the UICC. Any internal technical realisation of either the UICC or the ME is only specified where these are important for the verification of specific behaviour. Application specific details for applications residing on an UICC are specified in the respective application specific documents.

# 1	Scope

The present document provides application behavioural tests for a UE to interoperability with dedicated 3GPP network operations. Where there shall be a description for each compliant implementation including the:

- the test case specific setting of the USIM;

- the applicability of each test case;

- the test configurations;

- the conformance requirement(s) and reference(s) to the related core specifications;

- the test purposes; and

- a brief description of the test procedure and the specific acceptance criteria.

The present document is applicable for MEs supporting a non-removable UICC only. MEs that allow access to the physical card interface as defined in ETSI TS 102 221 [8] or ETSI TS 102 671 [29] shall be tested in accordance to TS 31.121 [2].

This document shall provide alternative verification and testing approaches for test cases initially defined in TS 31.121 [2]. If no ETSI or 3GPP defined alternative method to verify an existing requirement is available or if the result generated by the proposed verification method is less reliable, this has to be clearly stated in the test description.

Due to possible limitations in profile handling and updating it is not intended to use the test case defined within this document with 2G only MEs or MEs implemented in accordance to Rel-12 or earlier.

A 3GPP ME may support functionality that is not required by 3GPP, but the requirements to do so are outside of the scope of 3GPP. The present document does not contain tests for features that are out of scope of 3GPP.

In the present document, unless explicitly stated otherwise, for Rel-13 onwards the term E-UTRAN implicitly refers to E-UTRAN in WB-S1 mode. E-UTRAN in NB-S1 mode is always explicitly referred to as NB-IoT.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 31.121: "UICC-terminal interface; Universal Subscriber Identity Module (USIM) application test specification".

[3]	3GPP TS 38.508-1: "5GS; User Equipment (UE) conformance specification; Part 1: Common test environment".

[4]	3GPP TS 36.508: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); Common test environments for User Equipment (UE) conformance testing".

[5]	3GPP TS 34.108: "Common test environments for User Equipment (UE) conformance testing".

[6]	3GPP TS 34.229-1: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 1: Protocol conformance specification".

[7]	ISO/IEC 9646-7: "Information technology - Open Systems Interconnection - Conformance testing methodology and framework - Part 7: Implementation Conformance Statements".

[8]	ETSI TS 102 221: "UICC-Terminal interface; Physical and logical characteristics".

[9]	GSMA TS.48: "Generic eUICC Test Profile for Device Testing".

[10]	ETSI TS 103 666-1: "Smart Secure Platform (SSP); Part 1: General characteristics".

[11]	ETSI TS 103 666-2: "Smart Secure Platform (SSP); Part 2: Integrated SSP (iSSP) characteristics".

[12]	ETSI TS 103 666-3: "Smart Secure Platform (SSP); Part 3: Embedded SSP (eSSP) Type 1 characteristics".

[13]	802.11-2016: "IEEE Standard for Information technology—Telecommunications and information exchange between systems Local and metropolitan area networks—Specific requirements - Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

[14]	ETSI TS 102 225: "Secured packet structure for UICC based applications".

[15]	ETSI TS 102 226: "Remote APDU structure for UICC based applications".

[16]	3GPP TS 27.007: "AT command set for User Equipment (UE) ".

[17]	3GPP TS 31.130: "(U)SIM Application Programming Interface (API); (U)SIM API for Java™Card".

[18]	3GPP TS 22.011: "Service accessibility".

[19]	3GPP TS 31.102: "Characteristics of the Universal Subscriber Identity Module (USIM) application".

[20]	3GPP TS 21.111: "USIM and IC card requirements".

[21]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[22]	Trusted Connectivity Alliance: "eUICC Profile Package: Interoperable Format Technical Specification".

[23]	ETSI TS 102 241: "UICC Application Programming Interface (UICC API) for Java Card™".

[24]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[25]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[26]	3GPP TS 31.111: "Universal Subscriber Identity Module (USIM) Application Toolkit (USAT)".

[27]	Void

[28]	ETSI TS 102 221: "UICC-Terminal interface; Physical and logical characteristics".

[29]	ETSI TS 102 671: "Machine to Machine UICC; Physical and logical characteristics".

[30]	GSMA SGP.22: "RSP Technical specification".

[31]	3GPP TS 24.008: "Mobile radio interface Layer 3 specification; Core network protocols; Stage 3".

[32]	3GPP TS 25.331: "Radio Resource Control (RRC); Protocol specification".

[33]	3GPP TS 31.101: "UICC-terminal interface; Physical and logical characteristics".

[34]	3GPP TS 23.003: "Numbering, addressing and identification".

[35]	3GPP TS 31.117: "Universal Subscriber Identity Module Application Toolkit (USAT) application behavioural test specification".

[36]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[37]	3GPP TS 38.331: "NR Radio Resource Control (RRC) protocol specification".

[38]	3GPP TS 24.526: "User Equipment (UE) policies for 5G System (5GS); Stage 3".

[39]	3GPP TS 23.501: "System architecture for the 5G System (5GS); Stage 2".

[40]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[41]	3GPP TS 23.122: "Non-Access-Stratum functions related to Mobile Station (MS) in idle mode".

[42]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN)".

[43]	3GPP TS 34.124-1: "User Equipment (UE) conformance specification; Part 1: Protocol conformance specification".

[44]	3GPP TS 22.030: "Man-Machine Interface (MMI) of the User Equipment (UE)".

[45]	3GPP TS 22.101: "Smart cards; ETSI numbering system for telecommunication application providers".

[46]	3GPP TS 23.086: "Advice of Charge (AoC) supplementary services; Stage 2".

[47]	3GPP TS 24.086: "Advice of Charge (AoC) supplementary services; Stage 3".

[48]	3GPP TS 22.024: "Description of Charge Advice Information (CAI)".

[49]	3GPP TS 24.080: "Mobile radio interface layer 3 supplementary services specification; Formats and coding".

[50]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE); Security architecture".

[51]	RFC 5480: "Elliptic Curve Cryptography Subject Public Key Information".

[52]	3GPP TS 22.220: "Universal Mobile Telecommunications System (UMTS); Service requirements for Home Node B (HNB) and Home eNode B (HeNB)".

[53]	3GPP TS 33.102: "3G security; Security architecture".

[54]	RFC 4187: "Extensible Authentication Protocol Method for 3rd Generation Authentication and Key Agreement (EAP-AKA)".

[55]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[56]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[57]	3GPP TS 33.203: "Technical Specification Group Services and System Aspects; 3G security; Access security for IP-based services".

[58]	3GPP TS 34.229-5: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Part 5: Protocol conformance specification using 5G System (5GS) ".

[59]	3GPP TS 31.105: "Characteristics of the Slice Subscriber Identity Module application".

[60]	ETSI TS 101 220 v18.2.0: "ETSI numbering system for telecommunication application providers".

[61]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1], TS 31.121 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

nrUSIM:	non-removable Universal Subscriber Identity Module, i.e. a USIM application or equivalent functionality embedded or integrated into a ME.

nrSSIM:	non-removable Slice Subscriber Identity Module, i.e. a SSIM application as defined in TS 31.105 [59] or equivalent functionality embedded or integrated into a ME.

TT:	Test Tool: collective term for requirements fulfilling but not clearly specified test environment

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

bx		Bit x of byte (leftmost bit is MSB)

Bn		Byte No. n

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], TS 31.121 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

CR	Conformance Requirement

EUT	Equipment Under Test

SA	Suitability Assessment

TT		Test Tool

## 3.4	Coding Conventions

For the purposes of the present document, the following coding conventions apply:

All lengths are presented in bytes, unless otherwise stated. Each byte B is represented by eight bits b8 to b1, where b8 is the most significant bit (MSB) and b1 is the least significant bit (LSB). In each representation, the leftmost bit is the MSB.

In the UICC, all bytes specified as RFU shall be set to '00' and all bits specifies as RFU shall be set to '0'. If the GSM and/or USIM application exists on a UICC or is built on a generic telecommunications card, then other values may apply for the non- GSM or non-USIM applications. The values will be defined in the appropriate specifications for such cards and applications. These bytes and bits shall not be interpreted by a Terminal in 3GPP session.

The coding of all data objects in the present document is according to ETSI TS 102 221 [8]. All data objects are BER-TLV except if otherwise defined.

## 3.5	Generic procedures for 5G-NR, E-UTRAN, IMS and NB-IoT

If a test case contains the statement "This test applies to UEs accessing 5G-NR", the procedures defined in TS 38.508-1 [3] shall be the basis for all performed procedures during the test. The procedures in TS 38.508-1 [3] clause 4.5 describe the default behaviour of a conformant UE regarding the specified protocols to be used for 5G-NR and the required procedures from the NAS. TT (NG-SS) and UE shall use the default configuration defined in clause 4.7 appropriate to the supported options (e.g. 'voice centric').

5G-NR generic procedure for 5G Registration on 3GPP access with IMS service is defined in Annex Z.1.

If a test case contains the statement "This test applies to UEs accessing NB", the procedures defined in TS 36.508 [4] shall be the basis for all performed procedures during the test. The procedures in TS 36.508 [4] clause 8.1.5 describe the default behaviour of a conformant UE regarding the specified protocols to be used for NB-IoT and the required procedures from the NAS.

If a test case contains the statement "This test applies to UEs accessing E-UTRAN", the procedures defined in TS 36.508 [4] shall be the basis for all performed procedures during the test. The procedures in TS 36.508 [4], clause 4.5 describe the default behaviour of a conformant UE regarding the specified protocols to be used for E-UTRAN and the GH

## 3.6	Table of optional features

Support of several features is optional or release dependent for the user equipment. However, if a UE states conformance with a specific 3GPP release, it is mandatory for the UE to support all mandatory functions of that release, as stated in table A.1.

The supplier of the implementation shall state the support of possible options in table A.1.

Table A.1: Options

| Item | Option | Status | Support | Mnemonic |
| --- | --- | --- | --- | --- |
| 1 | Support of CS | O |  | O_CS |
| 2 | Support of a feature requiring PIN2 entry (such as e.g. AoC or FDN) | O |  | O_PIN2_ENTRY_FEAT |
| 3 | Support of UTRAN access | N/A |  | O_UTRAN |
| 4 | Support of GERAN access | N/A |  | O_GERAN |
| 5 | Support of Fixed Dialling Numbers | O |  | O_FDN |
| 6 | Support of Advice of Charge Charging | O |  | O_AoCC |
| 7 | Support of Higher Priority PLMN selector with Access Technology service | O |  | O_HPLMNwACT |
| 8 | Support of local phonebook | O |  | O_LOCAL_PB |
| 9 | Support of global phonebook | C001 |  | O_GLOBAL_PB |
| 10 | Support of storing received Class 2 Short Messages in the USIM | O |  | O_STORE_CLASS2_SMS |
| 11 | Support of MMS | O |  | O_MMS |
| 12 | Support of usage of MMS related data stored on the USIM | C002 |  | O_MMS_USIM_DATA |
| 13 | Supported of unselected user MMS connectivity parameters | O |  | O_NO_USER_MMS_CONF_SELEC |
| 14 | Support of MMS notification storage on the USIM | O |  | O_MMS_NOTIF_STORAGE |
| 15 | Support of ACL | O |  | O_ACL |
| 16 | Support of SDN | O |  | O_SDN |
| 17 | Support of numerical entry of PLMN codes in EF PLMNwACT | O |  | O_EFPLMNwACT_NUM_ENTRY |
| 18 | ME does support speech call | O |  | O_SPEECH_CALL |
| 19 | ME support PIN MMI strings | O |  | O_PIN_MMI_STRING |
| 20 | ME does support eFDD | O |  | O_eFDD |
| 21 | ME does support eTDD | O |  | O_eTDD |
| 22 | ME does support CSG list handling (for E-UTRA) | O |  | O_CSG_LIST |
| 23 | ME supports SM-over-IP-receiver | O |  | O_SM-OVER-IP_RECEIVER |
| 24 | ME supports reading SMS' stored in EFSMS on the USIM if USIM and ISIM are present | O |  | O_READ_USIM-EF_SMS_IF_USIM+ISIM |
| 25 | ME supports reading SMS' stored in EF SMS on the ISIM if USIM and ISIM are present | O |  | O_READ_ISIM-EF_SMS_IF_USIM+ISIM |
| 26 | ME can store more than 1000 text messages | O |  | O_LARGE_SMS_STORAGE |
| 27 | Support for multiple PDN connections | O |  | O_MULTIPLE_PDN |
| 28 | ME does support CSG (for UTRA) | O |  | O_CSG |
| 29 | Support of manual CSG selection | O |  | O_MANUAL_CSG_SELECTION |
| 30 | Support of PS | O |  | O_PS |
| 31 | ME does support display | O |  | O_DISPLAY |
| 32 | ME does support keypad | O |  | O_KEYPAD |
| 33 | ME supports E-UTRA Disabling Allowed for EMM cause #15 | O |  | O_DISABLE_EUTRA_EMM_CAUSE#15 |
| 34 | ME supports Override NAS Signalling Low Priority | O |  | O_OVERRIDE_NAS_SLP |
| 35 | ME supports T3245 timer | O |  | O_T3245 |
| 36 | ME supports Override Extended Access Barring | O |  | O_OVERRIDE_EAB |
| 37 | ME does support NB-IoT | O |  | O_NB-IoT |
| 38 | ME maintains a list of PLMN-specific attempt counters | O |  | O_PLMN_ATTEMPT_COUNTER |
| 39 | ME does support deactivation of the UICC in PSM. | O |  | O_PSM_DEAC_UICC |
| 40 | ME does support deactivation of the UICC during extended DRX | O |  | O_eDRX_DEAC_UICC |
| 41 | ME does support the UICC suspension mechanism in PSM. | O |  | O_PSM_SUSPEND_UICC |
| 42 | ME does support the UICC suspension mechanism during extended DRX | O |  | O_eDRX_SUSPEND_UICC |
| 43 | Support of 5G Core Network | O |  | O_5G_CN |
| 44 | Support of 5G New Radio access | O |  | O_5G_NR |
| 45 | Support of URSP by USIM | O |  | O_URSP_BY_USIM |
| 46 | ME supports SUPI as Network Access Identifier (NSI, GLI or GCI) | O |  | O_SUPI_NAI |
| 47 | Support of E-UTRAN access | O |  | O_E-UTRAN |
| 48 | Support of RSP(SGP.22) | O |  | O_RSP22 |
| 49 | Support of AT+CSIM | O |  | O_AT+CSIM |
| 50 | ME supports non-removable UICC only (see NOTE 1) | M |  | O_NON-REMOVABLE_UICC_ONLY |
| 51 | Support of UICC and USIM API for Java Card (see NOTE 2) | O |  | O_JAVA_CARD_API |
| 52 | Support of USAT functionality (see NOTE 3) | O |  | O_USAT |
| 53 | Support of CAG | O |  | pc_CAG |
| 54 | Support of multiple registrations by USIM | O |  | O_multregs_by_USIM |
| 55 | ME does support voice centric mode | O |  | pc_VOICE_CENTRIC |
| 56 | ME does support SSIM during NSSAA procedure | O |  | O_SSIM |

| C001 | If ((A.1/18 is supported) AND (A.1/31 is supported) AND (A.1/32 is supported)) THEN M, ELSE O |
| --- | --- |
| C002 | If (A.1/11 is NOT supported) THEN N/A, ELSE M |
| NOTE 1: | 'ME supports non-removable UICC only' means that access to the physical card interface as defined in ETSI TS 102 221 is not available |
| NOTE 2: | The UE shall claim to support the Java Card API if test relevant functions as defined in Annex A, clause A.2 are supported. |
| NOTE 3: | The support of the USAT functionalities as expected here requires the support of the UICC API defined in ETSI TS 102 241 [23] and the USIM API defined in TS 31.130 [17] |

## 3.7	Applicability

### 3.7.1	Applicability to user equipment

The applicability to user equipment supporting the non-removable USIM is specified in table B.1.

Test cases where no verification of APDUs or transferred data, timing, or checks on file (DF/EF) content is required, may refer to tests defined in TS 31.121 [2]. Regardless of references to complete tests, test purposes, conformance requirements or test methods from TS 31.121 [2] the applicability of the individual test cases is defined within the present document.

Tests where the implicit verification of conformance requirements is not considered sufficient on its own require additional (explicit) verification methods. The support of additional verification methods by the EUT has to be declared in accordance to table A.2 (see clause 3.7.2). Test sequence specific declarations of methods required to be supported are listed in the Applicability table – Table B.1.

### 3.7.2	Supported additional explicit verification methods

The support of additional verification methods is optional for an nrUICC operated device (EUT). As the implicit verification of test results is not sufficient in some test cases the support of an explicit testing option and the provisioning of an interface for file contents verification improves test coverage. The UE manufacturer shall declare the support of possible testable options listed in Table A.2. This declaration is used for the suitability assessment of the conformance requirement (CR) per test case.

Table A.2: Test Options Declaration

| Item | Option | Status | Support | Mnemonic |
| --- | --- | --- | --- | --- |
| 1 | Support of Toolkit Test Events (see note) | O |  | O_Toolkit_Test_Events |
| 2 | Support of seamless test APDU logging via Baseband (see note) | O |  | O_Seamless_APDU_Logging |
| 3 | Interface for file contents verification | O |  | O_File_Contents_Verification |
| NOTE: The support of the SSP Test Tool Interface (ETSI TS 103 834) is handled like the support of seamless test APDU logging via Baseband or for Test Toolkit Event based testing. Specific information may be added to test cases where needed. |  |  |  |  |

For details on these options see clauses 4.1.3, to 4.1.6 of the present document.

### 3.7.3	Applicability of the individual tests

Table B.1 lists the optional, conditional or mandatory features for which the supplier of the implementation states the support. As pre-condition the supplier of the implementation shall state the support of possible options in table A.1.

The "Release XY ME" columns shows the status of the entries as follows:

The following notations, defined in ISO/IEC 9646-7 [7], are used for the status column:

M	mandatory – the capability is required to be supported.

O	optional – the capability may be supported or not.

N/A	not applicable – in the given context, it is impossible to use the capability.

X	prohibited (excluded) – there is a requirement not to use this capability in the given context.

O.i	qualified optional – for mutually exclusive or selectable options from a set. "i" is an integer which identifies a unique group of related optional items and the logic of their selection which is defined immediately following the table.

Ci	conditional – the requirement on the capability ("M", "O", "X" or "N/A") depends on the support of other optional or conditional items. "i" is an integer identifying an unique conditional status expression which is defined immediately following the table. For nested conditional expressions, the syntax "IF … THEN (IF … THEN … ELSE…) ELSE …" shall be used to avoid ambiguities.

The "Additional test case execution recommendation" column shows the status of the entries as follows:

A	applicable - the test is applicable according to the corresponding entry in the "Rxx ME" column

R	redundant – the test has to be considered as redundant when the corresponding E-UTRAN/EPC related test of the present document has been validated and successfully executed. In that case the requirement may be verified by means of the E-UTRAN/EPC functionality only.

AERi	Additional test case Execution Recommendation – with respect to the above listed definitions of ("A") and ("R") the test is applicable ("A") or redundant ("R") depending on the support of other optional or conditional items. "i" is an integer identifying a unique conditional status expression which is defined immediately following the table. For nested conditional expressions, the syntax "IF ... THEN (IF ... THEN ... ELSE...) ELSE ..." shall be used to avoid ambiguities.

References to items

For each possible item answer (answer in the support column) there exists a unique reference, used, for example, in the conditional expressions. It is defined as the table identifier, followed by a solidus character "/", followed by the item number in the table. If there is more than one support column in a table, the columns shall be discriminated by letters (a, b, etc.), respectively.

EXAMPLE:	A.1/4 is the reference to the answer of item 4 in table A.1.

## 3.8	Applicability table

Table B.1: Applicability of tests

| Test# | Title | from Rel | to Rel (see note) | Applicability | Network Dependency | Test Option | Support |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | Subscription related tests |  |  |  |  |  |  |
| 5.1 | IMSI / TMSI handling |  |  |  |  |  |  |
| 5.1.6 | UE identification by short IMSI when accessing E-UTRAN/EPC | Rel-13 |  | C001 | E-USS OR NB-SS | E.04 |  |
| 5.1.7 | UE identification by short IMSI using 2 digit MNC when accessing E-UTRAN/EPC | Rel-13 |  | C001 | E-USS OR NB-SS | I.01 |  |
| 5.1.8 | UE identification after changed IMSI with service "EMM Information" not available | Rel-13 |  | C001 | E-USS OR NB-SS | I.01 |  |
| 5.1.9 | UE identification by GUTI when using USIM with service "EMM Information" not available | Rel-13 |  | C001 | E-USS OR NB-SS | I.01 |  |
| 5.1.10 | UE identification by GUTI when using USIM with service "EMM Information" available | Rel-13 |  | C001 | E-USS OR NB-SS | E.04 |  |
| 5.2 | Access Control handling |  |  |  |  |  |  |
| 5.2.2 | Access Control information handling for E-UTRAN/EPC | Rel-13 |  | C003 | E-USS | E.04 |  |
| 5.2.3 | Access Control information handling for NB-IoT | Rel-13 |  | C002 | NB-SS | E.04 |  |
| 5.3 | Handling subscription identifier privacy for 5G |  |  |  |  |  |  |
| 5.3.1 | SUCI calculation by ME using null scheme | Rel-15 |  | C004 | NG-SS | E.04 |  |
| 5.3.2 | SUCI calculation by ME using Profile B | Rel-15 |  | C004 | NG-SS | I.01 |  |
| 5.3.3 | UE identification by SUCI during initial registration – SUCI calculation by USIM using profile B | Rel-15 |  | C004 | NG-SS | E.04 |  |
| 5.3.4 | UE identification by SUCI in response to IDENTITY REQUEST message | Rel-15 |  | C004 | NG-SS | E.04 |  |
| 5.3.5 | UE identification by SUCI in response to IDENTITY REQUEST message with T3519 timer expiry | Rel-15 |  | C004 | NG-SS | I.01 |  |
| 5.3.6 | UE identification by SUCI in response to IDENTITY REQUEST message and AUTHENTICATION REJECT | Rel-15 |  | C004 | NG-SS | I.01 |  |
| 5.3.7 | SUCI calculation by the ME using null scheme – missing parameters for subscription identifier privacy support by the USIM | Rel-15 |  | C004 | NG-SS | E.04 |  |
| 5.3.8 | UE identification by 5G-GUTI – Last Registered TAI stored on USIM | Rel-15 |  | C004 | NG-SS | E.04 |  |
| 5.3.9 | UE identification by 5G-GUTI – Last Registered TAI stored by ME | Rel-15 |  | C004 | NG-SS | E.04 |  |
| 5.3.10 | UE identification after SUPI is changed | Rel-15 |  | C004 | NG-SS | E.04 |  |
| 5.3.11 | SUCI calculation by ME using Profile A | Rel-15 |  | C004 | NG-SS | E.04 |  |
| 5.3.12 | UE identification by SUCI during initial registration – SUCI calculation by USIM using profile A | Rel-15 |  | C004 | NG-SS | E.04 |  |
| 5.3.13 | SUCI calculation by ME using null scheme – no Protection Scheme Identifier provisioned in the USIM | Rel-15 |  | C004 | NG-SS | E.04 |  |
| 5.3.14 | SUCI calculation by ME using null scheme – no Home Network Public Key for supported protection scheme provisioned in the USIM | Rel-15 |  | C004 | NG-SS | E.04 |  |
| 5.3.15 | SUCI calculation by ME using null scheme with the E-UTRAN/EPC UICC | Rel-15 |  | C004 | NG-SS | I.01 |  |
| 5.3.16 | SUCI calculation by ME using the lower priority protection scheme when the higher priority protection scheme is not supported by the ME | Rel-15 |  | C004 | NG-SS | E.04 |  |
| 5.3.17 | SUCI calculation by ME using Profile B with compressed Home Network Public Key | Rel-15 |  | C004 | NG-SS | E.04 |  |
| 5.4 | Unified Access Control information handling for 5G-NR |  |  |  |  |  |  |
| 5.4.1 | Unified Access Control – Access identity 0, no access identities indicated by USIM | Rel-15 |  | C004 | NG-SS | E.04 |  |
| 5.4.1A | Unified Access Control – Access identity 0, no access identities indicated by USIM, Access Category 8 | Rel-15 |  | C005 | NG-SS | E.04 |  |
| 5.4.2 | Unified Access Control – Access Identity 1 – MPS indicated by USIM | Rel-15 |  | C004 | NG-SS | E.04 |  |
| 5.4.2A | Unified Access Control – Access Identity 1 – MPS indicated by USIM, Access Category 8 | Rel-15 |  | C005 | NG-SS | E.04 |  |
| 5.4.3 | Unified Access Control Access Identity 1 – no MPS indication by USIM and SUPI not changed | Rel-15 |  | C004 | NG-SS | E.04 |  |
| 5.4.4 | Unified Access Control Access Identity 1 – no MPS indication by USIM and SUPI is changed | Rel-15 |  | C004 | NG-SS | E.04 |  |
| 5.4.5 | Unified Access Control – Access Identity 2 – MCS indicated by USIM | Rel-15 |  | C004 | NG-SS | E.04 |  |
| 5.4.5A | Unified Access Control – Access Identity 2 – MCS indicated by USIM | Rel-15 |  | C005 | NG-SS | E.04 |  |
| 5.4.6 | Unified Access Control – Access Identity 2 – no MCS indication by USIM and SUPI is not changed | Rel-15 |  | C004 | NG-SS | E.04 |  |
| 5.4.7 | Unified Access Control – Access Identity 2 – no MCS indication by USIM and SUPI is changed | Rel-15 |  | C004 | NG-SS | E.04 |  |
| 5.4.8 | Unified Access Control – Access Identities 11 and 15 indicated by USIM | Rel-15 |  | C004 | NG-SS | E.04 |  |
| 5.4.8A | Unified Access Control – Access Identities 11 and 15 indicated by USIM, Access Category 8 | Rel-15 |  | C005 | NG-SS | E.04 |  |
| 5.4.9 | Unified Access Control – Access Identities 12, 13 and 14 indicated by USIM | Rel-15 |  | C004 | NG-SS | E.04 |  |
| 5.4.9A | Unified Access Control – Access Identities 12, 13 and 14 indicated by USIM, Access Category 8 | Rel-15 |  | C005 | NG-SS | E.04 |  |
| 5.4.10 | Unified Access Control – Operator-Defined Access Category | Rel-15 |  | C004 | NG-SS | E.04 |  |
| 5.4.11 | Unified Access Control – Operator-Defined Access Categories, no change in SUPI | Rel-15 |  | C004 | NG-SS | E.04 |  |
| 5.4.12 | Unified Access Control – Operator-Defined Access Categories, SUPI change | Rel-15 |  | C004 | NG-SS | E.04 |  |
| 5.5 | Handling of operator controlled features |  |  |  |  |  |  |
| 5.5.1 | Display of registered 5G PLMN name from USIM | Rel-15 |  | C006 | NG-SS | I.01 |  |
| 5.5.2 | Display of registered 5G PLMN name from ME | Rel-15 |  | C006 | NG-SS | I.01 |  |
| 5.6 | Handling subscription identifier privacy for 5G - SUPI type in NAI format |  |  |  |  |  |  |
| 5.6.1 | SUCI calculation by ME using null scheme | Rel-16 |  | C007 | NG-SS | E.07 |  |
| 5.6.2 | UE identification by SUCI during initial registration – SUCI calculation by USIM using profile A | Rel-16 |  | C007 | NG-SS | E.04 |  |
| 5.6.3 | UE identification by SUCI during initial registration – SUCI calculation by USIM using profile B | Rel-16 |  | C007 | NG-SS | E.04 |  |
| 5.6.4 | UE identification after SUPI is changed | Rel-16 |  | C007 | NG-SS | E.04 |  |
| 5.6.5 | UE identification by SUCI during initial registration – SUCI calculation by ME using profile A | Rel-16 |  | C007 | NG-SS | E.04 |  |
| 5.6.6 | UE identification by SUCI during initial registration – SUCI calculation by USIM using profile B | Rel-16 |  | C007 | NG-SS | E.04 |  |
| 6 | Security related Tests |  |  |  |  |  |  |
| 6.1 | PIN handling |  |  |  |  |  |  |
| 6.1.1 | Entry of PIN | Rel-13 |  | M |  | E.04 |  |
| 6.1.2 | Change of PIN | Rel-13 |  | M |  | E.04 |  |
| 6.1.3 | Unblock PIN | Rel-13 |  | C008 |  | E.04 |  |
| 6.1.4 | Entry of PIN2 | Rel-13 |  | C009 |  | E.04 |  |
| 6.1.5 | Change of PIN2 | Rel-13 |  | C009 |  | E.04 |  |
| 6.1.6 | Unblock PIN2 | Rel-13 |  | C010 |  | E.04 |  |
| 6.1.7 | Replacement of PIN | Rel-13 |  | O |  | E.04 |  |
| 6.1.8 | Change of Universal PIN | Rel-13 |  | O |  | E.04 |  |
| 6.1.9 | Unblock Universal PIN | Rel-13 |  | O |  | E.04 |  |
| 6.1.10 | Entry of PIN on multi-verification capable UICCs | Rel-13 |  | O |  | E.04 |  |
| 6.1.11 | Change of PIN on multi-verification capable UICCs | Rel-13 |  | O |  | E.04 |  |
| 6.1.12 | Unblock PIN on multi-verification capable UICCs | Rel-13 |  | O |  | E.04 |  |
| 6.1.13 | Entry of PIN2 on multi-verification capable UICCs | Rel-13 |  | O |  | E.04 |  |
| 6.1.14 | Change of PIN2 on multi-verification capable UICCs | Rel-13 |  | O |  | E.04 |  |
| 6.1.15 | Unblock PIN2 on multi-verification capable UICCs | Rel-13 |  | O |  | E.04 |  |
| 6.1.16 | Replacement of PIN with key reference "07" | Rel-13 |  | O |  | E.04 |  |
| 7 | PLMN related tests |  |  |  |  |  |  |
| 7.1 | FPLMN handling |  |  |  |  |  |  |
| 7.1.4 | Adding FPLMN to the forbidden PLMN list when accessing E-UTRAN | Rel-13 |  | C001 | E-USS OR NB-SS | E.04 |  |
| 7.1.5 | UE updating forbidden PLMNs when accessing E-UTRAN | Rel-13 |  | C001 | E-USS OR NB-SS | E.04 |  |
| 7.1.6 | UE deleting forbidden PLMNs when accessing E-UTRAN | Rel-13 |  | C001 | E-USS OR NB-SS | E.04 |  |
| 7.1.8 | Updating the Forbidden PLMN list after receiving non-integrity protected reject message – E-UTRAN | Rel-13 |  | C001 | E-USS OR NB-SS | E.04 |  |
| 7.2 | User controlled PLMN selector handling |  |  |  |  |  |  |
| 7.2.1 | UE updating the User controlled PLMN selector list | Rel-13 |  | C011 |  | E.07 |  |
| 7.2.5 | UE updating the User controlled PLMN selector list for E-UTRAN | Rel-13 |  | C011 AND C012 |  | E.07 |  |
| 7.2.8 | UE recognising the priority order of the User controlled PLMN selector list with the same access technology – E-UTRAN in NB-S1 mode | Rel-14 |  | C011 AND C002 | NB-SS | E.03 |  |
| 7.2.9 | UE recognising the priority order of the User controlled PLMN selector list using the ACT preference – E-UTRAN in WB-S1/E-UTRAN in NB-S1 | Rel-14 |  | C011 AND C012 AND C002 | E_USS AND NB-SS | E.03 |  |
| 7.3 | Operator controlled PLMN selector handling |  |  |  |  |  |  |
| 7.3.3 | UE recognising the priority order of the Operator controlled PLMN selector list when accessing E-UTRAN | Rel-13 |  | C001 | E-USS OR NB-SS | E.03 |  |
| 7.3.4 | UE recognising the priority order of the User controlled PLMN selector over the Operator controlled PLMN selector list – E-UTRAN | Rel-13 |  | C001 | E-USS OR NB-SS | E.03 |  |
| 7.3.5 | UE recognising the priority order of the Operator controlled PLMN selector list when accessing E-UTRAN in NB-S1 mode | Rel-14 |  | C002 | NB-SS | E.03 |  |
| 7.3.6 | UE recognising the priority order of the User controlled PLMN selector over the Operator controlled PLMN selector list – E-UTRAN in NB-S1 mode | Rel-14 |  | C011 AND C002 | NB-SS | E.03 |  |
| 7.3.7 | UE recognising the priority order of the Operator controlled PLMN selector list using the ACT preference - E-UTRAN in NB-S1/ E-UTRAN in WB-S1 mode | Rel-14 |  | C012 AND C002 | E-USS AND NB-SS | E.03 |  |
| 7.4 | Higher priority PLMN search handling |  |  |  |  |  |  |
| 7.4.3 | UE recognising the search period of the Higher priority PLMN – E-UTRAN | Rel-13 |  | C001 | E-USS OR NB-SS | E.03 |  |
| 7.4.7 | UE recognising the search period of the higher priority PLMN – NG-RAN | Rel-15 |  | C004 | NG-SS | E.03 |  |
| 8 | Subscription independent tests |  |  |  |  |  |  |
| 8.1 | Phone book procedures |  |  |  |  |  |  |
| 8.1.2 | Update of the Phonebook Synchronisation Counter (PSC) | Rel-13 |  | C012 |  | E.07 |  |
| 8.1.3 | Handling of BCD number/ SSC content extension | Rel-13 |  | C013 |  | E.07 |  |
| 8.1.4 | Phonebook selection | Rel-13 |  | C014 |  | E.07 |  |
| 8.1.5 | Local Phonebook handling | Rel-13 |  | C012 |  | E.07 |  |
| 8.2 | Short message handling report |  |  |  |  |  |  |
| 8.2.4B | Correct storage of an SM on the UICC | Rel-13 |  | C015 | E-USS | FFS |  |
| 8.2.5 | Correct reading of a SM on the USIM if USIM and ISIM are present | Rel-14 |  | C016 |  | FFS |  |
| 8.2.6 | Correct reading of a SM on the ISIM if USIM and ISIM are present | Rel-15 |  | C019 |  | FFS |  |
| 8.2.7 | Correct storage of an SM on the UICC | Rel-13 |  | C002 | NB-SS | FFS |  |
| 9 | USIM service handling |  |  |  |  |  |  |
| 9.1 | Access Point Name Control List handling |  |  |  |  |  |  |
| 9.1.4 | Access Point Name Control List handling for terminals supporting ACL connected to E-UTRAN/EPC | Rel-13 |  | C020 | E-USS | I.01 (E.03) |  |
| 10 | CSG list handling |  |  |  |  |  |  |
| 10.1 | CSG list handling for E-UTRA |  |  |  |  |  |  |
| 10.1.1 | Automatic CSG selection in E-UTRA with CSG list on USIM, success | Rel-13 |  | C021 | E-USS | E.04 |  |
| 10.1.2 | Automatic CSG selection in E-UTRA with CSG list on USIM, removal of CSG ID from the USIM | Rel-13 |  | C021 | E-USS | E.06 |  |
| 10.1.3 | Manual CSG selection in E-UTRA with CSG list on USIM, success | Rel-13 |  | C022 | E-USS | E.06 |  |
| 10.1.4 | Manual CSG selection in E-UTRA with CSG list on USIM, rejected | Rel-13 |  | C022 | E-USS | E.06 |  |
| 10.1.5 | CSG selection in E-UTRA with no CSG list on USIM, no IMSI change | Rel-13 |  | C022 | E-USS | I.01 |  |
| 10.1.6 | CSG selection in E-UTRA with no CSG list on USIM, with IMSI change | Rel-13 |  | C022 | E-USS | I.01 |  |
| 10.1.7 | Manual CSG selection without display restrictions in E-UTRA with ACSG list and OCSG list on USIM | Rel-13 |  | C022 | E-USS | E.04 |  |
| 10.1.8 | Manual CSG selection with display restrictions in E-UTRA with ACSG list and OCSG list on USIM | Rel-13 |  | C022 | E-USS | E.04 |  |
| 10.2 | CSG list handling for UTRA |  |  |  |  |  |  |
| 11 | NAS security context parameter handling |  |  |  |  |  |  |
| 11.1 | NAS security context parameter handling when service "EMM Information" is available | Rel-13 |  | C001 | E-USS OR NB-SS | E.04 |  |
| 11.2 | NAS security context parameter handling when service "EMM Information" is not available, no IMSI change | Rel-13 |  | C001 | E-USS OR NB-SS | E.04 |  |
| 11.3 | NAS security context parameter handling when service "EMM Information" is not available, IMSI changed | Rel-13 |  | C001 | E-USS OR NB-SS | E.04 |  |
| 11.4 | EPS NAS Security Context Storage | Rel-13 |  | C001 | E-USS OR NB-SS | E.04 |  |
| 12 | Non Access Stratum (NAS) Configuration parameter handling |  |  |  |  |  |  |
| 13 | UICC interface during PSM |  |  |  |  |  |  |
| 13.1 | UICC interface in PSM handling for E-UTRAN – No UICC deactivation in PSM | Rel-13 | Rel-13 | C023 | E-USS OR NB-SS | see note 1 |  |
|  |  | Rel-14 |  |  |  |  |  |
| 13.2 | UICC interface in PSM handling for E-UTRAN – PSM not accepted by E-USS/NB-SS | Rel-13 | Rel-13 | C023 | E-USS OR NB-SS | see note 1 |  |
|  |  | Rel-14 |  |  |  |  |  |
| 13.4 | UICC interface in PSM for E-UTRAN – SUSPEND UICC | Rel-14 |  | C023 | E-USS OR NB-SS | see note 1 |  |
| 14 | UICC interface during eDRX |  |  |  |  |  |  |
| 14.1 | UICC interface during eDRX for E-UTRAN – eDRX is not supported by the UICC | Rel-13 | Rel-13 | C024 | E-USS OR NB-SS | see note 1 |  |
|  |  | Rel-14 |  |  |  |  |  |
| 14.2 | UICC interface during eDRX for E-UTRAN – eDRX is not accepted by E-USS/NB-SS | Rel-13 | Rel-13 | C024 | E-USS OR NB-SS | see note 1 |  |
|  |  | Rel-14 |  |  |  |  |  |
| 14.3 | UICC interface during eDRX for E-UTRAN – UICC deactivation during eDRX | Rel-13 | Rel-13 | C024 | E-USS OR NB-SS | FFS |  |
|  |  | Rel-14 |  |  |  |  |  |
| 14.4 | UICC interface during eDRX for E-UTRAN– SUSPEND UICC | Rel-14 |  | C025 | E-USS OR NB-SS | FFS |  |
| 15 | Authentication procedure and NAS security context handling for 5G |  |  |  |  |  |  |
| 15.1 | Authentication procedure for EAP-AKA' |  |  |  |  |  |  |
| 15.1.1 | Authentication procedure for EAP-AKA' - Authentication is successful | Rel-15 | Rel-15 | C004 | NG-SS | E.07 |  |
| 15.1A.1 | Authentication procedure for EAP-AKA' - Authentication is successful | Rel-16 |  | C004 | NG-SS | E.04 (E.03) |  |
| 15.1.2 | Authentication procedure for EAP-AKA' – Authentication is successful - GSM UICC | Rel-15 |  | C004 | NG-SS | I.01 |  |
| 15.1.3 | Authentication procedure for EAP-AKA' – AUTN fails on the USIM | Rel-15 |  | C004 | NG-SS | E.04 |  |
| 15.1.4 | Authentication procedure for EAP-AKA' - after SUPI is changed | Rel-15 |  | C004 | NG-SS | I.01 |  |
| 15.2 | Authentication procedure for 5G AKA |  |  |  |  |  |  |
| 15.2.1 | Authentication procedure for 5G AKA - Authentication is successful | Rel-15 | Rel-15 | C004 | NG-SS | E.04 (E.03) |  |
| 15.2A.1 | Authentication procedure for 5G AKA - Authentication is successful | Rel-16 |  | C004 | NG-SS | E.04 (E.03) |  |
| 15.2.2 | Authentication procedure for 5G AKA – Authentication is successful - GSM UICC | Rel-15 |  | C004 | NG-SS | I.01 (E.04) |  |
| 15.2A.2 | Authentication procedure for 5G AKA – Authentication is successful - GSM UICC | Rel-16 | Rel-15 | C004 | NG-SS | I.01 |  |
| 15.2.3 | Authentication procedure 5G AKA – AUTN fails on the USIM | Rel-15 |  | C004 | NG-SS | E.04 |  |
| 15.2.4 | Authentication procedure for 5G AKA - after SUPI is changed | Rel-15 |  | C004 | NG-SS | I.01 |  |
| 15.2.5 | Authentication procedure for registrations 3GPP access 5G AKA - Authentication is successful in different PLMNs | Rel-16 |  | C028 | NG-SS | E.04 |  |
| 16 | UE Route Selection Policy (URSP) procedure |  |  |  |  |  |  |
| 16.1 | Pre-configured URSP rules |  |  |  |  |  |  |
| 16.1.1 | Support for URSP by USIM | Rel-16 |  | C026 | NG-SS | E.04 (E.03) |  |
| 16.1.2 | Support for URSP by ME | Rel-16 |  | C026 | NG-SS | E.04 (E.03) |  |
| 16.1.3 | Support of Signalled URSP | Rel-16 |  | C026 | NG-SS | E.04 (E.03) |  |
| 17 | CAG list handling |  |  |  |  |  |  |
| 17.1 | CAG list handling for 5G |  |  |  |  |  |  |
| 17.1.1 | Automatic CAG selection with preconfigured CAG list on USIM | Rel-17 |  | C027 | NG-SS | I.01 |  |
| 18 | NSSAA procedure with SSIM |  |  |  |  |  |  |
| 18.1 | SSIM management procedures |  |  |  |  |  |  |
| 18.1.1 | SSIM identification, S-NSSAI list request | Rel-18 |  | C029 | NG-SS |  |  |
| 18.2 | SSIM commands |  |  |  |  |  |  |
| 18.2.1 | AUTHENTICATE command, Identity EAP-Request/EAP-Response messages | Rel-18 |  | C029 | NG-SS |  |  |
| 18.2.2 | AUTHENTICATE command, other EAP-Request/EAP-Response messages | Rel-18 |  | C029 | NG-SS |  |  |
| 18.2.3 | AUTHENTICATE command, EAP-Success/EAP-Failure messages | Rel-18 |  | C029 | NG-SS |  |  |
| NOTE: Blank entries indicate the latest valid release at the time of publication of this specification |  |  |  |  |  |  |  |
| NOTE 1: Available methods are probably not sufficient. E.g. verification of electrical behaviour is needed |  |  |  |  |  |  |  |

Table B.2: Applicability conditions

| Condition# | Info | Mnemonic |
| --- | --- | --- |
| C001 | IF A.1/20 OR A.1/21 OR A.1/37 THEN M ELSE N/A | -- pc_eFDD OR pc_eTDD OR pc_NB |
| C002 | IF A.1/37 THEN M ELSE N/A | -- pc_NB |
| C003 | IF (A.1/20 OR A.1/21) AND A.1/27 THEN M ELSE N/A | -- (pc_eFDD OR pc_eTDD) AND pc_Multiple_PDN |
| C004 | IF A.1/43 AND A.1/44 THEN M ELSE N/A | -- pc_5GC AND pc_NR |
| C005 | IF A.1/43 AND A.1/44 AND A.1/47 THEN M ELSE N/A | -- pc_5GC AND pc_NR AND pc_inactiveState |
| C006 | IF A.1/43 AND A.1/44 AND A.1/31 THEN M ELSE N/A | -- pc_5GC AND pc_NR AND O_Display |
| C007 | IF A.1/43 AND A.1/44 AND A.1/46 THEN M ELSE N/A | -- pc_5GC AND pc_NR AND O_SUPI_NAI |
| C008 | IF A.1/19 THEN "Expected Sequence A" M ELSE "Expected Sequence B" M | -- O_PIN_MMI_Strings |
| C009 | IF A.1/2 THEN M ELSE N/A | -- O_PIN2_ENTRY_FEAT |
| C010 | IF A1/2 AND A.1/19 THEN "Expected Sequence A" M | -- (O_PIN2_ENTRY_FEAT AND O_PIN_MMI_Strings) |
| C011 | IF A.1/17 THEN M ELSE N/A | -- O_EFPLMNwACT_numerical entry |
| C012 | IF A.1/8 THEN M ELSE N/A | -- O_Local_PB |
| C013 | IF A.1/9 THEN M ELSE N/A | -- O_Global_PB |
| C014 | IF A.1/8 AND A.1/9 THEN M ELSE N/A | -- O_Local_PB AND O_Global_PB |
| C015 | IF (A.1/10 AND A.1/23 AND (A.1/20 OR A.1/21)) THEN M ELSE N/A | -- O_Store_Received_SMS AND pc_SM-over-IP receiver AND (pc_eFDD OR pc_eTDD) |
| C016 | IF A.1/24 THEN M ELSE N/A | -- pc_USIM_EF_SMS_reading_support_if_USIM_ISIM both present |
| C017 | IF A.1/11 AND A.1/13 THEN M ELSE N/A | -- O_MMS AND O_NO_USER_MMS_CONF_SELEC |
| C018 | IF A.1/11 AND A.1/14 THEN M ELSE N/A | -- O_MMS AND O_MMS_NOTIF_STORAGE |
| C019 | IF A.1/25 THEN M ELSE N/A | -- pc_ISIM_EF_SMS_reading_support_if_USIM_ISIM both present |
| C020 | IF A.1/15 AND (A.1/20 OR A.1/21) THEN M ELSE N/A | -- O_ACL AND (pc_eFDD OR pc_eTDD) |
| C021 | IF (A.1/20 OR A.1/21) AND A.1/22 THEN M ELSE N/A | -- (pc_eFDD OR pc_eTDD) AND pc_Allowed_CSG_list |
| C022 | IF (A.1/20 OR A.1/21) AND A.1/22 AND A.1/29 THEN M ELSE N/A | -- (pc_eFDD OR pc_eTDD) AND pc_Allowed_CSG_list AND pc_manual_CSG_selection |
| C023 | IF (A.1/20 OR A.1/21 OR A.1/37) AND A.1/39 THEN M ELSE N/A | -- (pc_eFDD OR pc_eTDD OR pc_NB) AND O_PSM_DEAC_UICC |
| C024 | IF (A.1/20 OR A.1/21 OR A.1/37) AND A.1/40 THEN M ELSE N/A | -- (pc_eFDD OR pc_eTDD OR pc_NB) AND O_eDRX_DEAC_UICC |
| C025 | IF (A.1/20 OR A.1/21 OR A.1/37) AND A.1/42 THEN M ELSE N/A | -- (pc_eFDD OR pc_eTDD OR pc_NB) AND O_eDRX_SUSPEND_UICC |
| C026 | IF A.1/43 AND A.1/44 AND A.1/45 THEN M ELSE N/A | -- pc_5GC AND pc_NR AND O_URSP_by_USIM |
| C027 | IF A.1/43 AND A.1/44 AND A.1/49 THEN M ELSE N/A | -- pc_5GC AND pc_NR AND pc_CAG |
| C028 | IF A.1/43 AND A.1/44 AND A.1/54 THEN M ELSE N/A | -- pc_5GC AND pc_NR AND O_multregs_by_USIM |
| C029 | IF A.1/43 AND A.1/44 AND A.1/56 | -- pc_5GC AND pc_NR AND O_SSIM |

| Table B.3: Test option |  |  |  |
| --- | --- | --- | --- |
| Option# | Meaning | Info |
| I.01 | Implicit verification | Only used if no explicit verification is needed within the test sequence |
| E.01 | Explicit verification, method A.2/1 only |  |
| E.02 | Explicit verification, method A.2/2 only |  |
| E.03 | Explicit verification, method A.2/3 only |  |
| E.04 | Explicit verification, method A.2/1 or A.2/2 |  |
| E.05 | Explicit verification, method A.2/1 or A.2/3 |  |
| E.06 | Explicit verification, method A.2/2 or A.2/3 |  |
| E.07 | Explicit verification, method A.2/1 or A.2/2 or A2.3 |  |

## 3.9	Rules for the representation of number systems and strings

For the representation of decimal numbers, binary numbers, hexadecimal numbers and strings in text of the present document the following rules apply:

| Convention | Description |
| --- | --- |
| nnnnn | Decimal number, e.g.: PIN value or phone number |
| 'b' | Single digit binary number ('0' or '1') |
| 'bbb'/'bbb bbbb' | Multiple digit binary number, e.g.: '010' or '111 0101' |
| 'bbbb bbbb' | 8-bit binary number, e.g.: '0110 0001' (separator between MSB and LSB) |
| 'hh' | Single octet hexadecimal number, e.g.: '2B' |
| 'hh hh .. hh' | Multi-octet hexadecimal number, e.g.; 'FF 00 .. C3' |
| "SSSS" | Character string, e.g.: "FDN111” |
| NOTE: Unless otherwise specified, a 'x' or 'X' present in a binary or hexadecimal number is "don't care" |  |

In addition to the enclosing quotation marks, bit values should be clearly identifiable as such by means of additional information. E.g.: 100 (bit 5-7) or CAG only bit: 1.

# 4	Test environment

## 4.1	Test environment description

### 4.1.1	General test environment

Without having the UICC-Terminal interface accessible a direct verification of APDU/data timing and contents is not possible. Thus, alternative implementations and methods will be used to provide sufficient confidence in the result obtained. The present document will not specify an authoritative test environment. The following figure shows a test environment that allows the verification of test results for UEs with an integrated and not removable UICC/USIM (nrUSIM).

The present document shall provide a test environment and test methods that allow the verification of test results for UEs with an integrated and not removable UICC/USIM (nrUSIM).

Figure 4.1: General test environment

Figure 4.1 gives an overview on how a test case shall be executed.

Based on the identified test purpose and the related conformance requirements an appropriate test sequence is defined. The test itself can be split into three phases:

In the preparation phase the initial set-up for the test case is performed. Test specific data is transferred to the nrUSIM. E.g. by provisioning a test specific profile.

In the execution phase the test procedure is performed. It has to be ensured that all steps defined in the test procedure are executed and that they are executed in order. The data generated during this execution is stored in the TT and/or test EFs in the file system within the nrUICC.

In the verification phase the data, procedures and processes identified and stored during execution are checked against given conformance requirements. A final verification of specific EF/DF contents might be required. The exchange of required data has to be ensured, even though the required data transfer is not necessarily listed in the test procedure.

### 4.1.2	Example - test environment for implicit testing

Implicit testing within the scope of the present document describes an end-to-end test method where the UICC/USIM file system of the DUT is prepared to contain specific data. During test execution the data transferred from the DUT to the TT is checked and compared to the expected result or value respectively.

Example:

-	During preparation phase specific content for the EFIMSI is updated in the nrUSIM;

-	A registration to the network shall be executed using this value;

-	The TT compares the initial value sent with the value returned during registration.

To ensure that no pre-configured value is used, the values to be verified in such a test procedure shall be generated as described in clause 4.3 of the present document.

NOTE:	A test toolkit applet can be used for testing USAT specific test cases in TS 31.117 [35].

Figure 4.2: Test environment for implicit testing

### 4.1.3	Example - test environment for contents verification

Contents verification within the scope of the present document describes a procedure that allows the TT to compare contents of EFs/DFs available on the EUT with expected values defined in the present document.

Example:

-	During preparation phase specific content for the EFFPLMN is updated in the nrUSIM;

-	During execution of the test the FPLMN value is modified;

-	As the FPLMN value stored in EFFPLMN is not transferred to the TT during test execution;

-	The TT performs a read procedure on EFFPLMN at test case end (not necessarily part of the test procedure);

-	The TT compares the value read with the expected value stored in the TT.

The contents verification method described here is not applicable when a verification of EF/DF contents needs to be performed at a time other than the preparation or the verification phase.

NOTE:	It is not expected that a reading procedure on EF contents can be performed whilst the test procedure is executed.

The test environment needed to perform contents verification is identical to the test environment shown for implicit testing.

### 4.1.4	Example - test environment for seamless testing

Seamless testing within the scope of the present document relies on monitoring of data transfer between ME and nrUSIM traced by a software interface between nrUSIM and baseband implemented by the ME vendor.

The logged communication can be transferred to the TT and will be used to determine if conformance requirements are met (see example in Annex D).

NOTE:	A test toolkit applet may be used for testing USAT specific test cases in TS 31.117 [35]. Refer to Annex B.1 for examples of test EFs required when using the test toolkit applet.

Figure 4.3: Test environment for seamless testing

### 4.1.5	Example – test environment for test toolkit events based testing

Test toolkit events based testing within the scope of the present document is applicable for UEs supporting the required USIM application toolkit functionality. A toolkit applet is installed onto the nrUSIM, capable of handling test events internal to the card runtime environment and the applets to monitor APDUs received at the nrUICC.

NOTE:	The same test toolkit applet may be used for testing USAT specific test cases in TS 31.117 [35]. Refer to Annex B.2 for examples of test EFs required when using the test toolkit applet.

The logged events can be stored in a test EF during the test execution phase and transferred to the TT during the verification phase to determine if conformance requirements are met.

ME may provide an interface for tester to monitor APDU communication during test execution through the TT.

Figure 4.4: Test environment for test toolkit events based testing

Please see the required "Test configuration state" in Annex N and the "Examples of test configuration usage" in Annex O of ETSI TS 102 221 [8]

### 4.1.6	Example – test environment for UEs with ETSI eSSP/iSSP

For testing UEs with an USIM functionality provided by an eSSP/iSSP as defined in ETSI TS 103 666-x specifications a test tool interface as defined in ETSI TS 103 834 has to be supported. For the usage within this specification the SSP test tool interface is handled like the test environment for seamless or for test toolkit event based testing. Specific execution requirements or setups may be added to this specification when required. Adaption needed to operate tests on UEs with an eSSP/iSSP, like e.g., an APDU gate are out of scope of the present document.

Figure 4.5: Test environment for UEs with ETSI eSSP/iSSP

## 4.2	Requirements to the EUT and the test environment

### 4.2.1	General Requirements

All tests defined in the subsequent clauses apply to UEs operating an nrUSIM, what implies that the UICC-Terminal interface cannot be exposed to a TT. Depending on the test method used, an explicit verification of APDUs or data sent via the UICC-Terminal interface might not be possible. If explicit verification of APDUs, data or file contents is needed to verify a conformance requirement the implementation and applicability has to be declared (see also clause 3.7.1).

The tests are not applicable for UEs providing a UICC-Terminal interface in accordance to interface form factors defined in ETSI TS 102 221 [28] or ETSI TS 102 671 [29].

The following sequence of tests confirms:

a)	the usage of the test specific UICC/USIM data;

b)	the correct interpretation of data read from the USIM (Universal Subscriber Identification Module) by the ME;

c)	the correct writing of data to the USIM by the ME;

d)	the initiation of appropriate procedures by the ME;

e)	the correct execution of functions

All tests apply to the USIM application on the UICC or an equivalent application implemented in accordance to the ETSI SSP specifications ETSI TS 103 666-1 [10], ETSI TS 103 666-2 [11] and ETSI TS 103 666-3 [12]

### 4.2.2	Requirements to the UE (EUT) – supported interfaces

The EUT has to support interfaces and administration methods to allow the TT or the User to set the initial conditions defined for the test cases. Where the EUT may be solely the UE or the UE connected to an interface device.

NOTE:	The connection of the UE and an interface device is set up similar to what is described in the Companion Device scenario in GSMA SGP.22 [30], but not limited to remote SIM provisioning functionality.

Suggested interfaces:

•	Wi-Fi (IEEE 802.11-2016 [13])

•	USB (USB-IF, USB 2.0 or higher)

The UE has to support at least one of the following data transfer methods:

•	RFM and OTA via ETSI TS 102 225 [14] and ETSI TS 102 226 [15]

•	AT commands as defined in TS 27.007 [16]

•	JavaTMCard as defined in TS 31.130 [17]

Even though the availability of at least one of the suggested interfaces and one of the suggested data transfer method is recommended, interfaces and methods for UICC/USIM preparation for testing purposes are to be provided by the UE vendor and are out of scope of the present document.

### 4.2.3	Supported RATs

UEs tested in accordance to the present document shall support any 3GPP defined RATs. Test cases defined in the present document may contain RAT or RAN specific conformance requirements and methods of test. The applicability of the individual test cases can be determined by using Table B.1.

Tests that would require 2G network access (GERAN) are out of scope of the present document.

### 4.2.4	Initial and final procedure steps

Initial and final procedure steps are out of scope of the current specification to not explicitly exclude methods that might be used to get a UE set up at test case start or to have it 'cleaned up' at the end of a test (if required).

The testing person has to ensure that the UE has installed test specific the UICC and USIM data in accordance to the definitions of the particular test (see note). The UE has to be accessible and is brought into a specific reception mode if required by the test case. It has to be ensured that interfaces that are used to verify file data or a specific functionality do not interfere or block the operation of the test procedure as defined within this specification.

For verification purposes an interaction with the UE is allowed even after ending the defined test procedure. If such interaction takes place it has to be reported to the TT. A verification of conformance requirements that is based on data or information that is generated during this post-procedure interaction has to be identifiable as a post process, but can be used for the result generation.

NOTE:	For all EFs, DFs and ADFs building the UICC and USIM where no data is explicitly defined in the test or by reference, an appropriate test value can be used (e.g.: values from GSMA TS.48 [9]).

## 4.3	Random value generation

To ensure that the EUT provides EF values read from the UICC/USIM, identified EFs are to be used with random EF content.

If the test definition requires random test values to be used, the EF content is generated on test case initialization. An EF containing random values always is specified in the initial condition clause of the test case. At least the EF coding is showing one or more placeholders (characters x to z, highlighted in red) where a generated random value has to be inserted. To avoid ambiguities a codomain (D) of possible values is defined for each random character.

Example:

EFIMSI (International Mobile Subscriber Identity)

Logically:

24681357x

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 05 | 29 | 64 | 18 | 53 | x7 | FF | FF | FF |

D:	 x ∈ {0x0; …; 0x9}

Explanation:

The random IMSI can become any logical value from 246813570 to 246813579.

NOTE:	the verification method using the generated random value is described in Annex A.4

EF content randomization (e.g. last digit) is limited to the elementary files EFIMSI, EFLOCI (files), EFRouting_Indicator and Home Network Public Key identifier in the EFSUCI_Cal_Info and it is done for a limited number of network-dependent test cases only.

## 4.4	Suitability assessment

### 4.4.1	Suitability assessment in present test procedures

For test procedures defined within the present document the step table holds a separate column for the "suitability assessment", abbreviated as "SA". It will show the identifier of any conformance requirement (CR) where gaining suitable verification requires the usage of an optional test method. Conformance requirements that can be verified by "implicit" methods, e.g. the usage of a specific value is sufficiently verified if the test proceeds correctly with the next step or if the expected value can be seen on the air interface, will not be listed in the SA column.

### 4.4.2	Suitability assessment in test descriptions

For each CR listed in the SA column of a test procedure available in the present document additional information about the SA shall be provided with the description of the CR verification.

Tests purposes, conformance requirements and/or procedures defined in TS 31.121 [2] may be referenced. Modifications needed to adopt a test from TS 31.121 [2] to use the random value generation for the implicit testing will not be explicitly mentioned. If it becomes necessary to add suitable test methods from Table A.2 to a particular CR this shall be done by adding the related information to the related CR in the Acceptance criteria clauses available in the present document.

### 4.4.3	Suitability assessment in the test protocol

Whenever a test is executed using additional methods to generate a result the method and the gained result shall be noted in the test protocol. If none of the indicated optional test methods is supported, it shall be clearly noted that the no verification took place. A test step to be verified by an explicit method that could not be verified has to be marked as "Inconclusive". In case any test step is marked as "Inconclusive", the overall test case verdict shall be set to "Inconclusive".

### 4.4.4	Justification of additional measures

Even if implicit verification is recognized as a suitable and appropriate verification method, additional methods to explicitly verify file contents and/or the file handling shall be used if supported. The verification result and the used verification method shall be noted in the test report. In case this additional measure leads to a deviating result, the result of the explicit check defined to this test case applies.

Conformance requirements that can be verified with such an additional measure are marked with an ‘O’ in the SA column. Possible optional verification methods are noted in the acceptance criteria for each test case if applicable.

## 4.5	Definition of default values

### 4.5.1	Introduction

#### 4.5.1.1	Installation, provisioning or modification methods for EFs and DFs

Installation, provisioning or modification methods for EFs and DFs defined in the UICCs/USIMs used for testing or in the test cases are out of scope of the present document. Respective methods have to be provided by the UE vendor.

#### 4.5.1.2	TS.48 Version and usage

Unless stated differently in the test description or in the specific UICC definition TS.48 eSIM GTP v5.0 is used

The usage of file values defined in GSMA TS.48 [9] does not imply that remote SIM provisioning as defined in GSMA or profiles as defined by the TCA have to be supported by the nrUSIM.

### 4.5.2	Definition of the Default UICC

The Default UICC for the present document is build from TS.48 eSIM GTP referenced in clause 4.5.1.2 with the following exceptions:

EFAD (Administrative Data)

Logically:

Mode of operation:				normal operation

Additional information:			ciphering indicator feature disabled

Length of MNC in the IMSI:	3 digit

Coding:

| Byte | B1 | B2 | B3 | B4 |
| --- | --- | --- | --- | --- |
| Hex | 00 | 00 | 00 | 03 |

EFUST (USIM Service Table)

Logically:

| Service n°1: |  | Local Phone Book | available |
| --- | --- | --- | --- |
| Service n°2: |  | Fixed Dialling Numbers (FDN) | available |
| Service n°6: |  | Barred Dialling Numbers (BDN) | available |
| Service n°17: |  | Group Identifier Level 1 | not available |
| Service n°18: |  | Group Identifier Level 2 | not available |
| Service n°20: |  | User controlled PLMN selector with Access Technology | available |
| Service n°27: |  | GSM Access | available |
| Service n°33: |  | shall be set to '1' | available |
| Service n°34: |  | Enabled Services Table | available |
| Service n°85: |  | EPS Mobility Management Information | available |
| Service n°86: |  | Allowed CSG Lists and corresponding indications | available |

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary | xx1x xx11 | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxxx xxxx | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | B12 |
|  | xxxx xxxx | xxxx xxxx | xx11 xxxx | 0000 0xxx |

### 4.5.3	Definition of FDN UICC

The values of the FDN UICC are identical to the values of the Default UICC defined in clause 4.5.2 of the present document with the following exceptions:

#### EFEST (Enable Service Table)

Logically:

| Service n°1: |  | Fixed Dialling Numbers (FDN) | activated |
| --- | --- | --- | --- |
| Service n°2: |  | Barred Dialling Numbers (BDN | deactivated |
| Service n°3: |  | APN Control List (ACL) | deactivated |

Coding:

| Byte: | B1 |
| --- | --- |
| Binary | xxxx x001 |

#### EFFDN (Fixed Dialling Numbers)

Logically:

Record 1:

Length of alpha identifier:	6 characters;

Alpha identifier:				"FDN111";

Length of BCD number:		6 bytes;

TON and NPI:				Telephony and International;

Dialled number:				+1357924680;

CCI2:							None;

Ext2:							None.

Coding for record 1:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 46 | 44 | 4E | 31 | 31 | 31 | 06 | 91 | 31 | 75 | 29 | 64 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
|  | 08 | FF | FF | FF | FF | FF | FF | FF |

Record 2:

Length of alpha identifier:	6 characters;

Alpha identifier:				"FDN222";

Length of BCD number:		4 bytes;

TON and NPI:				Telephony and International;

Dialled number:				24680;

CCI2:							None;

Ext2:							None.

Coding for record 2:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 46 | 44 | 4E | 32 | 32 | 32 | 04 | 81 | 42 | 86 | F0 | FF |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
|  | FF | FF | FF | FF | FF | FF | FF | FF |

Record 3:

Length of alpha identifier:	6 characters;

Alpha identifier:				"FDN333";

Length of BCD number:		11 bytes;

TON and NPI:				Telephony and International;

Dialled number:				+12345678901234567890;

CCI2:							None;

Ext2:							None.

Coding for record 3:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 46 | 44 | 4E | 33 | 33 | 33 | 0B | 91 | 21 | 43 | 65 | 87 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
|  | 09 | 21 | 43 | 65 | 87 | 09 | FF | FF |

EFECC (Emergency Call Codes)

Logically:

Emergency call code:					122;

Emergency call code alpha identifier:	"TEST";

Emergency call Service Category:		Mountain Rescue.

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 21 | F2 | FF | 54 | 45 | 53 | 54 | 10 |

EF/DF definitions provided in the Initial condition clause of a test case/test sequence take precedence over values defined as default.

### 4.5.4	Definition of E-UTRAN/EPC UICC

The values of the E-UTRAN/EPC UICC are identical to the values of the Default UICC defined in clause 4.5.2 of the present document with the following exceptions:

EFUST (USIM Service Table)

Logically:

Allow services up to Service n°120

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary | xx1x xx11 | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxxx xxxx | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | B12 | B13 | B14 | B15 |
|  | xxxx xxxx | xxxx xxxx | xx11 xxxx | xxxx xxxx | xxxx xxxx | xxxx xxxx | xxxx xxxx |

EFEPSLOCI (EPS Location Information)

Logically:

GUTI:							24608100010266431122

Last visited registered TAI:	246/081/0001

EPS update status:			not updated

Coding:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex: | 0B | F6 | 42 | 16 | 80 | 00 | 01 | 02 | 66 | 43 | 11 | 22 |
|  | B13 | B14 | B15 | B16 | B17 | B18 |
|  | 42 | 16 | 80 | 00 | 01 | 01 |

EFPLMNwACT (User Controlled PLMN Selector with Access Technology) is configured as defined in TS 31.121 [2] clause 4.8.3

EFOPLMNwACT (Operator Controlled PLMN Selector with Access Technology) is configured as defined in TS 31.121 [2] clause 4.8.4

EFACSGL (Allowed CSG Lists) is configured as defined in TS 31.121 [2] clause 4.8.5

EFCSGT (CSG Type) is configured as defined in TS 31.121 [2] clause 4.8.6

EFHNBN (Home (e)NodeB Name) is configured as defined in TS 31.121 [2] clause 4.8.7

EFEPSNSC (EPS NAS Security Context) is configured as defined in TS 31.121 [2] clause 4.8.8

EFNASCONFIG (Non Access Stratum Configuration) is configured as defined in TS 31.121 [2] clause 4.8.9

EF/DF definitions provided in the Initial condition clause of a test case/test sequence take precedence over values defined as default.

### 4.5.5	Definition of E-UTRAN/EPC ISIM-UICC

The E-UTRAN/EPC ISIM-UICC shall contain a USIM as defined in clause 4.5.4 of the present document and an ISIM for IMS access as defined in TS 31.121 [2], clause 4.5.4.

EF/DF definitions provided in the Initial condition clause of a test case/test sequence take precedence over values defined as default.

### 4.5.6	Definition of ACSGL/OCSGL E-UTRAN/EPC UICC

The values of the ACSGL/OCSGL E-UTRAN/EPC UICC are identical to the values of the Default UICC defined in clause 4.5.2 of the present document with the following exceptions:

EFUST (USIM Service Table)

Logically:

| Service n°90: |  | Operator CSG Lists and corresponding indications | available |
| --- | --- | --- | --- |
| Service n°92: |  | Support of CSG Display Control | available |

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary | xx1x xx11 | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxxx xxxx | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | B12 |
|  | xxxx xxxx | xxxx xxxx | xx11 xxxx | 0000 1x1x |

EFAD (Administrative Data)

Logically:

Mode of operation:				normal operation

Additional information:			ciphering indicator feature disabled

all available CSGs can be displayed without any restriction (b2)

Length of MNC in the IMSI:		3 digit

Coding:

| Byte | B1 | B2 | B3 | B4 |
| --- | --- | --- | --- | --- |
| Hex | 00 | 00 | 00 | 03 |

EFOCSGL (Operator CSG Lists)

For testing 2 CSG lists are defined and stored together in record one.

Logically:

1st CSG list

PLMN:		246 081 (MCC MNC)

1st CSG list	1st CSG Type indication			01

1st CSG list	1st CSG HNB Name indication	01

1st CSG list	1st CSG CSG ID:					01 (27bit)

1st CSG list	2nd CSG Type indication			05

1st CSG list	2nd CSG HNB Name indication	05

1st CSG list	2nd CSG CSG ID:				05 (27bit)

CSG display indicator:	All available CSG IDs can be displayed during a manual CSGselection 00

2nd CSG list

PLMN:		244 081 (MCC MNC)

2nd CSG list	1st CSG Type indication			07

2nd CSG list	1st CSG HNB Name indication	07

2nd CSG list	1st CSG CSG ID:					07 (27bit)

CSG display indicator:	All available CSG Ids can be displayed during a manual CSGselection 00

Coding:

| Byte | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | A0 | 18 | 80 | 03 | 42 | 16 | 80 | 81 | 06 | 01 | 01 | 00 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 00 | 00 | 3F | 81 | 06 | 05 | 05 | 00 | 00 | 00 | BF | 82 |
|  | B25 | B26 | B27 | B28 | B29 | B30 | B31 | B32 | B33 | B34 | B35 | B36 |
|  | 01 | 00 | A0 | 10 | 80 | 03 | 42 | 14 | 80 | 81 | 06 | 07 |
|  | B37 | B38 | B39 | B40 | B41 | B42 | B43 | B44 |
|  | 07 | 00 | 00 | 00 | FF | 82 | 01 | 00 |

EFOCSGT (Operator CSG Type)

Record 1:

Logically:	OMode ONE

Coding:

| Byte | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 89 | 13 | 80 | 00 | 4F | 00 | 4D | 00 | 6F | 00 | 64 | 00 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 65 | 00 | 20 | 00 | 4F | 00 | 4E | 00 | 45 | FF | FF | FF |
|  | B25 | B26 | B27 | B28 | B29 | B30 |
|  | FF | FF | FF | FF | FF | FF |

Record 2:

Logically:	OMode TWO

Coding:

| Byte | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 89 | 13 | 80 | 00 | 4F | 00 | 4D | 00 | 6F | 00 | 64 | 00 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 65 | 00 | 20 | 00 | 54 | 00 | 57 | 00 | 4F | FF | FF | FF |
|  | B25 | B26 | B27 | B28 | B29 | B30 |
|  | FF | FF | FF | FF | FF | FF |

Record 3:

Logically:	OMode THREE

Coding:

| Byte | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 89 | 17 | 80 | 00 | 4F | 00 | 4D | 00 | 6F | 00 | 64 | 00 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 65 | 00 | 20 | 00 | 54 | 00 | 48 | 00 | 52 | 00 | 45 | 00 |
|  | B25 | B26 | B27 | B28 | B29 | B30 |
|  | 45 | FF | FF | FF | FF | FF |

Record 4:

Logically:	OMode FOUR

Coding:

| Byte | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 89 | 15 | 80 | 00 | 4F | 00 | 4D | 00 | 6F | 00 | 64 | 00 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 65 | 00 | 20 | 00 | 46 | 00 | 4F | 00 | 55 | 00 | 52 | FF |
|  | B25 | B26 | B27 | B28 | B29 | B30 |
|  | FF | FF | FF | FF | FF | FF |

Record 5:

Logically:	Group FIVE

Coding:

| Byte | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 89 | 15 | 80 | 00 | 47 | 00 | 72 | 00 | 6F | 00 | 75 | 00 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 70 | 00 | 20 | 00 | 46 | 00 | 49 | 00 | 56 | 00 | 45 | FF |
|  | B25 | B26 | B27 | B28 | B29 | B30 |
|  | FF | FF | FF | FF | FF | FF |

Record 6:

Logically:	Group SIX

Coding:

| Byte | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 89 | 13 | 80 | 00 | 47 | 00 | 72 | 00 | 6F | 00 | 75 | 00 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 70 | 00 | 20 | 00 | 53 | 00 | 49 | 00 | 58 | FF | FF | FF |
|  | B25 | B26 | B27 | B28 | B29 | B30 |
|  | FF | FF | FF | FF | FF | FF |

Record 7:

Logically:	Group SEVEN

Coding:

| Byte | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 89 | 17 | 80 | 00 | 47 | 00 | 72 | 00 | 6F | 00 | 75 | 00 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 70 | 00 | 20 | 00 | 53 | 00 | 45 | 00 | 56 | 00 | 45 | 00 |
|  | B25 | B26 | B27 | B28 | B29 | B30 |
|  | 4E | FF | FF | FF | FF | FF |

Record 8:

Logically:	Group EIGHT

Coding:

| Byte | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 89 | 17 | 80 | 00 | 47 | 00 | 72 | 00 | 6F | 00 | 75 | 00 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 70 | 00 | 20 | 00 | 45 | 00 | 49 | 00 | 47 | 00 | 48 | 00 |
|  | B25 | B26 | B27 | B28 | B29 | B30 |
|  | 54 | FF | FF | FF | FF | FF |

All other records are empty.

EFOCSGT (Operator CSG Type)

Record 1:

Logically:	OMode ONE

Coding:

| Byte | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 89 | 13 | 80 | 00 | 4F | 00 | 4D | 00 | 6F | 00 | 64 | 00 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 65 | 00 | 20 | 00 | 4F | 00 | 4E | 00 | 45 | FF | FF | FF |
|  | B25 | B26 | B27 | B28 | B29 | B30 |
|  | FF | FF | FF | FF | FF | FF |

Record 2:

Logically:	OMode TWO

Coding:

| Byte | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 89 | 13 | 80 | 00 | 4F | 00 | 4D | 00 | 6F | 00 | 64 | 00 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 65 | 00 | 20 | 00 | 54 | 00 | 57 | 00 | 4F | FF | FF | FF |
|  | B25 | B26 | B27 | B28 | B29 | B30 |
|  | FF | FF | FF | FF | FF | FF |

Record 3:

Logically:	OMode THREE

Coding:

| Byte | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 89 | 17 | 80 | 00 | 4F | 00 | 4D | 00 | 6F | 00 | 64 | 00 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 65 | 00 | 20 | 00 | 54 | 00 | 48 | 00 | 52 | 00 | 45 | 00 |
|  | B25 | B26 | B27 | B28 | B29 | B30 |
|  | 45 | FF | FF | FF | FF | FF |

Record 4:

Logically:	OMode FOUR

Coding:

| Byte | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 89 | 15 | 80 | 00 | 4F | 00 | 4D | 00 | 6F | 00 | 64 | 00 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 65 | 00 | 20 | 00 | 46 | 00 | 4F | 00 | 55 | 00 | 52 | 00 |
|  | B25 | B26 | B27 | B28 | B29 | B30 |
|  | FF | FF | FF | FF | FF | FF |

Record 5:

Logically:	Group FIVE

Coding:

| Byte | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 89 | 15 | 80 | 00 | 47 | 00 | 72 | 00 | 6F | 00 | 75 | 00 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 70 | 00 | 20 | 00 | 46 | 00 | 49 | 00 | 56 | 00 | 45 | FF |
|  | B25 | B26 | B27 | B28 | B29 | B30 |
|  | FF | FF | FF | FF | FF | FF |

Record 6:

Logically:	Group SIX

Coding:

| Byte | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 89 | 13 | 80 | 00 | 47 | 00 | 72 | 00 | 6F | 00 | 75 | 00 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 70 | 00 | 20 | 00 | 53 | 00 | 49 | 00 | 58 | FF | FF | FF |
|  | B25 | B26 | B27 | B28 | B29 | B30 |
|  | FF | FF | FF | FF | FF | FF |

Record 7:

Logically:	Group SEVEN

Coding:

| Byte | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 89 | 17 | 80 | 00 | 47 | 00 | 72 | 00 | 6F | 00 | 75 | 00 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 70 | 00 | 20 | 00 | 53 | 00 | 45 | 00 | 56 | 00 | 45 | 00 |
|  | B25 | B26 | B27 | B28 | B29 | B30 |
|  | 4E | FF | FF | FF | FF | FF |

Record 8:

Logically:	Group EIGHT

Coding:

| Byte | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 89 | 17 | 80 | 00 | 47 | 00 | 72 | 00 | 6F | 00 | 75 | 00 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 70 | 00 | 20 | 00 | 45 | 00 | 49 | 00 | 47 | 00 | 48 | 00 |
|  | B25 | B26 | B27 | B28 | B29 | B30 |
|  | 54 | FF | FF | FF | FF | FF |

EFOHNBN (Operator Home (e)NodeB Name)

Record 1:

Logically:	Home ONE

Coding:

| Byte | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 80 | 11 | 80 | 00 | 48 | 00 | 6F | 00 | 6D | 00 | 65 | 00 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 20 | 00 | 4F | 00 | 4E | 00 | 45 | FF | FF | FF | FF | FF |
|  | B25 | B26 | B27 | B28 | B29 | B30 |
|  | FF | FF | FF | FF | FF | FF |

Record 2:

Logically:	Home TWO

Coding:

| Byte | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 80 | 11 | 80 | 00 | 48 | 00 | 6F | 00 | 6D | 00 | 65 | 00 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 20 | 00 | 54 | 00 | 57 | 00 | 4F | FF | FF | FF | FF | FF |
|  | B25 | B26 | B27 | B28 | B29 | B30 |
|  | FF | FF | FF | FF | FF | FF |

Record 3:

Logically:	Home THREE

Coding:

| Byte | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 80 | 15 | 80 | 00 | 48 | 00 | 6F | 00 | 6D | 00 | 65 | 00 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 20 | 00 | 54 | 00 | 48 | 00 | 52 | 00 | 45 | 00 | 45 | FF |
|  | B25 | B26 | B27 | B28 | B29 | B30 |
|  | FF | FF | FF | FF | FF | FF |

Record 4:

Logically:	Home FOUR

Coding:

| Byte | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 80 | 13 | 80 | 00 | 48 | 00 | 6F | 00 | 6D | 00 | 65 | 00 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 20 | 00 | 46 | 00 | 4F | 00 | 55 | 00 | 52 | FF | FF | FF |
|  | B25 | B26 | B27 | B28 | B29 | B30 |
|  | FF | FF | FF | FF | FF | FF |

Record 5:

Logically:	Home FIVE

Coding:

| Byte | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 80 | 13 | 80 | 00 | 48 | 00 | 6F | 00 | 6D | 00 | 65 | 00 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 20 | 00 | 46 | 00 | 49 | 00 | 56 | 00 | 45 | FF | FF | FF |
|  | B25 | B26 | B27 | B28 | B29 | B30 |
|  | FF | FF | FF | FF | FF | FF |

Record 6:

Logically:	Home SIX

Coding:

| Byte | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 80 | 11 | 80 | 00 | 48 | 00 | 6F | 00 | 6D | 00 | 65 | 00 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 20 | 00 | 53 | 00 | 49 | 00 | 58 | FF | FF | FF | FF | FF |
|  | B25 | B26 | B27 | B28 | B29 | B30 |
|  | FF | FF | FF | FF | FF | FF |

Record 7:

Logically:	Home SEVEN

Coding:

| Byte | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 80 | 15 | 80 | 00 | 48 | 00 | 6F | 00 | 6D | 00 | 65 | 00 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 20 | 00 | 53 | 00 | 45 | 00 | 56 | 00 | 45 | 00 | 4E | FF |
|  | B25 | B26 | B27 | B28 | B29 | B30 |
|  | FF | FF | FF | FF | FF | FF |

Record 8:

Logically:	Home EIGHT

Coding:

| Byte | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 80 | 15 | 80 | 00 | 48 | 00 | 6F | 00 | 6D | 00 | 65 | 00 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 20 | 00 | 45 | 00 | 49 | 00 | 47 | 00 | 48 | 00 | 54 | FF |
|  | B25 | B26 | B27 | B28 | B29 | B30 |
|  | FF | FF | FF | FF | FF | FF |

EF/DF definitions provided in the Initial condition clause of a test case/test sequence take precedence over values defined as default.

### 4.5.7	Definition of Non-Access Stratum Configuration UICC

In general, the values of the Non-Access Stratum Configuration UICC are identical to the values of the Default UICC. Non-Access Stratum Configuration UICC specific configurations, as defined in TS 31.121 [2], clause 4.7 apply.

EF/DF definitions provided in the Initial condition clause of a test case/test sequence take precedence over values defined as default.

### 4.5.8	Definition of Non-Access Stratum Configuration of E-UTRAN/EPC UICC

In general, the values of the Non-Access Stratum Configuration of E-UTRAN/EPC UICC are identical to the values of the Default UICC. Non-Access Stratum Configuration of E-UTRAN/EPC UICC specific configurations, as defined in TS 31.121 [2], clause 4.8 apply.

EF/DF definitions provided in the Initial condition clause of a test case/test sequence take precedence over values defined as default.

### 4.5.9	Definition of 5G-NR UICC

The values of the 5G-NR UICC are identical to the values of the Default UICC defined in clause 4.5.2 of the present document with the following exceptions:

EFUST (USIM Service Table)

Logically:

Allow services up to Service n°131

| Service n°122: |  | 5GS Mobility Management Information | available |
| --- | --- | --- | --- |
| Service n°123: |  | 5G Security Parameters | available |
| Service n°124: |  | Subscription identifier privacy support | available |
| Service n°125: |  | SUCI calculation by the USIM | not available |

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary: | xx1x xx11 | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxxx xxxx | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | … | B16 | B17 |  |  |
|  | xxxx xxxx | xxxx xxxx | xx11 xxxx | ... | xxx0 111x | 0000 0xxx |  |  |

EF5GS3GPPLOCI (5GS 3GPP location information)

Logically:

5G-GUTI:				FF FF FF FF FF FF FF FF FF FF

TAI:					246 081 000000

5GS update status:	5U2 NOT UPDATED

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | FF | FF | FF | FF | FF | FF | FF | FF |
|  | B9 | B10 | B11 | B12 | B13 | B14 | B15 | B16 |
|  | FF | FF | FF | FF | FF | 42 | 16 | 80 |
|  | B17 | B18 | B19 | B20 |
|  | 00 | 00 | 00 | 01 |

EFSUCI_Calc_Info (Subscription Concealed Identifier Calculation Information EF)

Logically:

Protection Scheme Identifier List data object

Protection Scheme Identifier 1 – ECIES scheme profile B

Key Index 1: 1

Protection Scheme Identifier 2 – ECIES scheme profile A

Key Index 2: 2

Protection Scheme Identifier 3 – null-scheme

Key Index 3: 0

Home Network Public Key List data object

Home Network Public Key 1 Identifier: 27

Home Network Public Key 1:

-	04 72 DA 71 97 62 34 CE 83 3A 69 07 42 58 67 B8 2E 07 4D 44 EF 90 7D FB 4B 3E 21 C1 C2 25 6E BC D1 5A 7D ED 52 FC BB 09 7A 4E D2 50 E0 36 C7 B9 C8 C7 00 4C 4E ED C4 F0 68 CD 7B F8 D3 F9 00 E3 B4

Home Network Public Key 2 Identifier: 30

Home Network Public Key 2:

-	5A 8D 38 86 48 20 19 7C 33 94 B9 26 13 B2 0B 91 63 3C BD 89 71 19 27 3B F8 E4 A6 F4 EE C0 A6 50

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | A0 | 06 | 02 | 01 | 01 | 02 | 00 | 00 |
|  | B9 | B10 | B11 | B12 | B13 | B14 | B15 | B16 |
|  | A1 | 6B | 80 | 01 | 1B | 81 | 41 | 04 |
|  | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 72 | DA | 71 | 97 | 62 | 34 | CE | 83 |
|  | B25 | B26 | B27 | B28 | B29 | B30 | B31 | B32 |
|  | 3A | 69 | 07 | 42 | 58 | 67 | B8 | 2E |
|  | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 |
|  | 07 | 4D | 44 | EF | 90 | 7D | FB | 4B |
|  | B41 | B42 | B43 | B44 | B45 | B46 | B47 | B48 |
|  | 3E | 21 | C1 | C2 | 25 | 6E | BC | D1 |
|  | B49 | B50 | B51 | B52 | B53 | B54 | B55 | B56 |
|  | 5A | 7D | ED | 52 | FC | BB | 09 | 7A |
|  | B57 | B58 | B59 | B60 | B61 | B62 | B63 | B64 |
|  | 4E | D2 | 50 | E0 | 36 | C7 | B9 | C8 |
|  | B65 | B66 | B67 | B68 | B69 | B70 | B71 | B72 |
|  | C7 | 00 | 4C | 4E | ED | C4 | F0 | 68 |
|  | B73 | B74 | B75 | B76 | B77 | B78 | B79 | B80 |
|  | CD | 7B | F8 | D3 | F9 | 00 | E3 | B4 |
|  | B81 | B82 | B83 | B84 | B85 | B86 | B87 | B88 |
|  | 80 | 01 | 1E | 81 | 20 | 5A | 8D | 38 |
|  | B89 | B90 | B91 | B92 | B93 | B94 | B95 | B96 |
|  | 86 | 48 | 20 | 19 | 7C | 33 | 94 | B9 |
|  | B97 | B98 | B99 | B100 | B101 | B102 | B103 | B104 |
|  | 26 | 13 | B2 | 0B | 91 | 63 | 3C | BD |
|  | B105 | B106 | B107 | B108 | B109 | B110 | B111 | B112 |
|  | 89 | 71 | 19 | 27 | 3B | F8 | E4 | A6 |
|  | B113 | B114 | B115 | B116 | B117 |
|  | F4 | EE | C0 | A6 | 50 |

EFRouting_Indicator (Routing Indicator EF)

Logically:

Routing Indicator: 17

Coding:

| Byte | B1 | B2 | B3 | B4 |
| --- | --- | --- | --- | --- |
| Hex | 71 | FF | 00 | 00 |

EF5GS3GPPNSC (5GS 3GPP Access NAS Security Context)

This file shall be available.

Logically:

ngKSI:										'07'	(no key available)

KAMF:											32 byte key, any value

Uplink NAS count:							'00'

Downlink NAS count:						'01'

Identifiers of selected NAS integrity
and encryption algorithms:				'01'

Identifiers of selected EPS NAS
integrity and encryption algorithms
for use after mobility to EPS:			'01'

Coding:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | … | B39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | A0 | 37 | 80 | 01 | 07 | 81 | 20 | xx | … | xx |
|  | B40 | B41 | B42 | B43 | B44 | B45 | B46 | B47 | B48 | B49 |
|  | 82 | 04 | 00 | 00 | 00 | 00 | 83 | 04 | 00 | 00 |
|  | B50 | B51 | B52 | B53 | B54 | B55 | B56 | B57 |
|  | 00 | 01 | 84 | 01 | 01 | 85 | 01 | 01 |

EF5GSN3GPPNSC (5GS non-3GPP Access NAS Security Context)

This file shall be available.

Logically:

ngKSI:										'07'	(no key available)

KAMF:											32 byte key, any value

Uplink NAS count:							'00'

Downlink NAS count:						'01'

Identifiers of selected NAS integrity
and encryption algorithms:				'01'

Identifiers of selected EPS NAS
integrity and encryption algorithms
for use after mobility to EPS:			'01'

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | … | B39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | A0 | 37 | 80 | 01 | 07 | 81 | 20 | xx | … | xx |
|  | B40 | B41 | B42 | B43 | B44 | B45 | B46 | B47 | B48 | B49 |
|  | 82 | 04 | 00 | 00 | 00 | 00 | 83 | 04 | 00 | 00 |
|  | B50 | B51 | B52 | B53 | B54 | B55 | B56 | B57 |
|  | 00 | 01 | 84 | 01 | 01 | 85 | 01 | 01 |

EF/DF definitions provided in the Initial condition clause of a test case/test sequence take precedence over values defined as default.

### 4.5.10	Definition of 5G-NR UICC – non-IMSI SUPI Type

The values of the 5G-NR UICC – non-IMSI SUPI Type are identical to the values of the 5G-NR UICC defined in clause 4.5.9 of the present document with the following exceptions:

EFAD (Administrative Data)

Logically:

Mode of operation:				normal operation

Additional information:			ciphering indicator feature disabled

Length of MNC in the IMSI:	0

Coding:

| Byte | B1 | B2 | B3 | B4 |
| --- | --- | --- | --- | --- |
| Hex | 00 | 00 | 00 | 00 |

EFUST (USIM Service Table)

Logically:

| Service n°130: |  | Support for SUPI of type NSI or GLI or GCI | available |
| --- | --- | --- | --- |

Coding:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary: | xx1x xx11 | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxxx xxxx | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | … | B16 | B17 |  |  |
|  | xxxx xxxx | xxxx xxxx | xx11 xxxx | ... | xxx0 111x | 0000 0x1x |  |  |

EFIMSI (IMSI)

This file shall not be available.

EFSUPI_NAI (SUPI as Network Access Identifier)

This file shall be available.

Logically:		00-00-5E-00-53-00@5gc.mnc012.mcc345.3gppnetwork.org

SUPI Type: 	GCI

Username: 	00-00-5E-00-53-00

Realm:		5gc.mnc012.mcc345.3gppnetwork.org

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 82 | 33 | 30 | 30 | 2D | 30 | 30 | 2D |
|  | B9 | B10 | B11 | B12 | B13 | B14 | B15 | B16 |
|  | 35 | 45 | 2D | 30 | 30 | 2D | 35 | 33 |
|  | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 2D | 30 | 30 | 40 | 35 | 67 | 63 | 2E |
|  | B25 | B26 | B27 | B28 | B29 | B30 | B31 | B32 |
|  | 6D | 6E | 63 | 30 | 31 | 32 | 2E | 6D |
|  | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 |
|  | 63 | 63 | 33 | 34 | 35 | 2E | 33 | 67 |
|  | B41 | B42 | B43 | B44 | B45 | B46 | B47 | B48 |
|  | 70 | 70 | 6E | 65 | 74 | 77 | 6F | 72 |
|  | B49 | B50 | B51 | B52 | B53 |
|  | 6B | 2E | 6F | 72 | 67 |

EFSUCI_Calc_Info (Subscription Concealed Identifier Calculation Information EF)

Logically:

Protection Scheme Identifier List data object

Protection Scheme Identifier 1 – ECIES scheme profile B

Key Index 1: 1

Protection Scheme Identifier 2 – ECIES scheme profile A

Key Index 2: 2

Protection Scheme Identifier 3 – null-scheme

Key Index 3: 0

Home Network Public Key List data object

Home Network Public Key 1 Identifier: 27

Home Network Public Key 1:

-	04 72 DA 71 97 62 34 CE 83 3A 69 07 42 58 67 B8 2E 07 4D 44 EF 90 7D FB 4B 3E 21 C1 C2 25 6E BC D1 5A 7D ED 52 FC BB 09 7A 4E D2 50 E0 36 C7 B9 C8 C7 00 4C 4E ED C4 F0 68 CD 7B F8 D3 F9 00 E3 B4

Home Network Public Key 2 Identifier: 30

Home Network Public Key 2:

-	5A 8D 38 86 48 20 19 7C 33 94 B9 26 13 B2 0B 91 63 3C BD 89 71 19 27 3B F8 E4 A6 F4 EE C0 A6 50

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | A0 | 06 | 02 | 01 | 01 | 02 | 00 | 00 |
|  | B9 | B10 | B11 | B12 | B13 | B14 | B15 | B16 |
|  | A1 | 6B | 80 | 01 | 1B | 81 | 41 | 04 |
|  | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 72 | DA | 71 | 97 | 62 | 34 | CE | 83 |
|  | B25 | B26 | B27 | B28 | B29 | B30 | B31 | B32 |
|  | 3A | 69 | 07 | 42 | 58 | 67 | B8 | 2E |
|  | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 |
|  | 07 | 4D | 44 | EF | 90 | 7D | FB | 4B |
|  | B41 | B42 | B43 | B44 | B45 | B46 | B47 | B48 |
|  | 3E | 21 | C1 | C2 | 25 | 6E | BC | D1 |
|  | B49 | B50 | B51 | B52 | B53 | B54 | B55 | B56 |
|  | 5A | 7D | ED | 52 | FC | BB | 09 | 7A |
|  | B57 | B58 | B59 | B60 | B61 | B62 | B63 | B64 |
|  | 4E | D2 | 50 | E0 | 36 | C7 | B9 | C8 |
|  | B65 | B66 | B67 | B68 | B69 | B70 | B71 | B72 |
|  | C7 | 00 | 4C | 4E | ED | C4 | F0 | 68 |
|  | B73 | B74 | B75 | B76 | B77 | B78 | B79 | B80 |
|  | CD | 7B | F8 | D3 | F9 | 00 | E3 | B4 |
|  | B81 | B82 | B83 | B84 | B85 | B86 | B87 | B88 |
|  | 80 | 01 | 1E | 81 | 20 | 5A | 8D | 38 |
|  | B89 | B90 | B91 | B92 | B93 | B94 | B95 | B96 |
|  | 86 | 48 | 20 | 19 | 7C | 33 | 94 | B9 |
|  | B97 | B98 | B99 | B100 | B101 | B102 | B103 | B104 |
|  | 26 | 13 | B2 | 0B | 91 | 63 | 3C | BD |
|  | B105 | B106 | B107 | B108 | B109 | B110 | B111 | B112 |
|  | 89 | 71 | 19 | 27 | 3B | F8 | E4 | A6 |
|  | B113 | B114 | B115 | B116 | B117 |
|  | F4 | EE | C0 | A6 | 50 |

EF/DF definitions provided in the Initial condition clause of a test case/test sequence take precedence over values defined as default.

### 4.5.11	Definition of 5G-NR UICC – support of Rel-16 features

The values of the 5G-NR UICC – support of Rel-16 features are identical to the values of the 5G-NR UICC defined in clause 4.5.9 of the present document with the following exceptions:

EFUST (USIM Service Table)

Logically:

Allow services up to Service n°136

| Service n°133: |  | 5G Security Parameters extended | available |
| --- | --- | --- | --- |
| Service n°136 |  | Support for multiple records of NAS security context storage for multiple registration | available |

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary: | xx1x xx11 | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxxx xxxx | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | … | B16 | B17 |  |  |
|  | xxxx xxxx | xxxx xxxx | xx11 xxxx | ... | xxx0 111x | 1xx1 xxx1 |  |  |

EF5GS3GPPNSC (5GS 3GPP Access NAS Security Context)

This file shall be available and shall contain two records (see 3GPP TS 31.102 [4]).

Logically:

First record:

ngKSI:										'07'	(no key available)

KAMF:											32 byte key, any value

Uplink NAS count:							'00'

Downlink NAS count:						'01'

Identifiers of selected NAS integrity
and encryption algorithms:				'01'

Identifiers of selected EPS NAS
integrity and encryption algorithms
for use after mobility to EPS:			'01'

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | … | B39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | A0 | 37 | 80 | 01 | 07 | 81 | 20 | xx | … | xx |
|  | B40 | B41 | B42 | B43 | B44 | B45 | B46 | B47 | B48 | B49 |
|  | 82 | 04 | 00 | 00 | 00 | 00 | 83 | 04 | 00 | 00 |
|  | B50 | B51 | B52 | B53 | B54 | B55 | B56 | B57 |
|  | 00 | 01 | 84 | 01 | 01 | 85 | 01 | 01 |

Second record

ngKSI:										'07'	(no key available)

KAMF:											32 byte key, any value

Uplink NAS count:							'00'

Downlink NAS count:						'01'

Identifiers of selected NAS integrity
and encryption algorithms:				'01'

Identifiers of selected EPS NAS
integrity and encryption algorithms
for use after mobility to EPS:			'01'

PLMN identifier:							'000000'

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | … | B39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | A0 | 3C | 80 | 01 | 07 | 81 | 20 | xx | … | xx |
|  | B40 | B41 | B42 | B43 | B44 | B45 | B46 | B47 | B48 | B49 |
|  | 82 | 04 | 00 | 00 | 00 | 00 | 83 | 04 | 00 | 00 |
|  | B50 | B51 | B52 | B53 | B54 | B55 | B56 | B57 | B58 | B59 |
|  | 00 | 01 | 84 | 01 | 01 | 85 | 01 | 01 | 86 | 03 |
|  | B60 | B61 | B62 |
|  | 00 | 00 | 00 |

EF5GSN3GPPNSC (5GS non-3GPP Access NAS Security Context)

This file shall be available and shall contain two records (see 3GPP TS 31.102 [4]).

Logically:

First record:

ngKSI:										'07'	(no key available)

KAMF:											32 byte key, any value

Uplink NAS count:							'00'

Downlink NAS count:						'01'

Identifiers of selected NAS integrity
and encryption algorithms:				'01'

Identifiers of selected EPS NAS
integrity and encryption algorithms
for use after mobility to EPS:			'01'

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | … | B39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | A0 | 37 | 80 | 01 | 07 | 81 | 20 | xx | … | xx |
|  | B40 | B41 | B42 | B43 | B44 | B45 | B46 | B47 | B48 | B49 |
|  | 82 | 04 | 00 | 00 | 00 | 00 | 83 | 04 | 00 | 00 |
|  | B50 | B51 | B52 | B53 | B54 | B55 | B56 | B57 |
|  | 00 | 01 | 84 | 01 | 01 | 85 | 01 | 01 |

Second record

ngKSI:										'07'	(no key available)

KAMF:											32 byte key, any value

Uplink NAS count:							'00'

Downlink NAS count:						'01'

Identifiers of selected NAS integrity
and encryption algorithms:				'01'

Identifiers of selected EPS NAS
integrity and encryption algorithms
for use after mobility to EPS:			'01'

PLMN identifier:							'000000'

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | … | B39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | A0 | 3C | 80 | 01 | 07 | 81 | 20 | xx | … | xx |
|  | B40 | B41 | B42 | B43 | B44 | B45 | B46 | B47 | B48 | B49 |
|  | 82 | 04 | 00 | 00 | 00 | 00 | 83 | 04 | 00 | 00 |
|  | B50 | B51 | B52 | B53 | B54 | B55 | B56 | B57 | B58 | B59 |
|  | 00 | 01 | 84 | 01 | 01 | 85 | 01 | 01 | 86 | 03 |
|  | B60 | B61 | B62 |
|  | 00 | 00 | 00 |

EF/DF definitions provided in the Initial condition clause of a test case/test sequence take precedence over values defined as default.

### 4.5.12	Definition of 5G-NR UICC – support of Rel-17 features

The values of the 5G-NR UICC – support of Rel-17 features are identical to the values of the 5G-NR UICC – support of Rel-16 features defined in clause 4.5.11 of the present document with the following exceptions:

EFUST (USIM Service Table)

Logically:

Allow services up to Service n°146

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary: | xx1x xx11 | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxxx xxxx | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | … | B16 | B17 | B18 | B19 |
|  | xxxx xxxx | xxxx xxxx | xx11 xxxx | ... | xxx0 111x | xxx1 xxx1 | xxxx xxxx | 0000 00xx |

### 4.5.13	Definition 5G-NR UICC – CAG support

In general, the values of the 5G-NR UICC – CAG support are identical to the values of the of 5G-NR UICC as defined in clause 4.5.12 in addition the following exceptions apply:

EFUST (USIM Service Table)

Logically:

| Service n°137: |  | Preconfigured CAG information list | available |
| --- | --- | --- | --- |

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary: | xx1x xx11 | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxxx xxxx | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | … | B16 | B17 | B18 | B19 |
|  | xxxx xxxx | xxxx xxxx | xx11 xxxx | ... | xxx0 111x | xxxx xxxx | xxxx xxx1 | 0000 00xx |

EFCAG (Pre-configured CAG information list EF)

Logically:

PLMN: 					244 083 (MCC MNC)

CAG only bit:			1

Range indication bit: 	1

CAG-ID range: 			'00 00 00 01' to '00 00 00 07'

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 00 | 0D | 0C | 42 | 34 | 80 | 03 | 00 | 00 | 00 |
|  | B11 | B12 | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
|  | 01 | 00 | 00 | 00 | 07 |

EF/DF definitions provided in the Initial condition clause of a test case/test sequence take precedence over values defined as default.

### 4.5.14	Definition of 5G-NR ISIM-UICC

The 5G-NR ISIM-UICC shall contain a USIM as defined in clause 4.5.9 of the present document and an ISIM for IMS access as defined in TS 31.121 [2], clause 4.5.3.

EF/DF definitions provided in the Initial condition clause of a test case/test sequence take precedence over values defined as default.

### 4.5.15	Definition of 5G-NR UICC – support of Rel-18 features

The values of the 5G-NR UICC – support of Rel-18 features are identical to the values of the 5G-NR UICC – support of Rel-17 features defined in clause 4.5.12 of the present document with the following exceptions:

EFUST (USIM Service Table)

Logically:

Allow services up to Service n°150

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary: | xx1x xx11 | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxxx xxxx | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | … | B16 | B17 | B18 | B19 |
|  | xxxx xxxx | xxxx xxxx | xx11 xxxx | ... | xxx0 111x | xxx1 xxx1 | xxxx xxxx | 00xx xxxx |

### 4.5.16	Definition of 5G-NR SSIM-UICC

The 5G-NR SSIM-UICC shall contain a SSIM as defined in TS 31.105 [59], with an AID value build from a 3GPP RID, and PIX coding as defined for the '3GPP SSIM' application in ETSI TS 101 220 [60], Annex E and a USIM as defined in clause 4.5.y of the present document.

EF/DF definitions provided in the Initial condition clause of a test case/test sequence take precedence over values defined as default.

EFEAPID (EAP Identifier)

Logically:

EAP Identifier:	verylongusername1@3gpp.com

Username:		verylongusername1

Realm:			3gpp.com

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 80 | 1A | 76 | 65 | 72 | 79 | 6C | 6F | 6E | 67 | 75 | 73 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 65 | 72 | 6E | 61 | 6D | 65 | 31 | 40 | 33 | 67 | 70 | 70 |
|  | B25 | B26 | B27 | B28 |
|  | 2E | 63 | 6F | 6D |

EFNSSAI (S-NSSAI List)

Logically:

Record 1:

S-NSSAI:		01 01 01 03 (SST: eMBB, SD: 010103)

Coding:

| Byte | B1 | B2 | B3 | B4 |
| --- | --- | --- | --- | --- |
| Hex | 01 | 01 | 01 | 03 |

Record 2:

S-NSSAI:		01 01 01 02 (SST: eMBB, SD: 010102)

Coding:

| Byte | B1 | B2 | B3 | B4 |
| --- | --- | --- | --- | --- |
| Hex | 01 | 01 | 01 | 02 |

## 4.6	Definition of default values for EFs commonly used with random value generation

### 4.6.1	EFIMSI with 3-digit MNC and random value

EFIMSI (International Mobile Subscriber Identity)

Logically:

2460813579135x

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 08 | 21 | 64 | 80 | 31 | 75 | 19 | 53 | Fx |

D:	 x ∈ {0x0; …; 0x9}

If the test/sequence is executed using an explicit test option as identified in Table A.2/1 or A.2/2, providing log data for all UICC-Terminal communication, x may be set to x=7.

### 4.6.2	EFIMSI "short", with 3-digit MNC and random value

EFIMSI (International Mobile Subscriber Identity)

Logically:

24608135x

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 05 | 29 | 64 | 80 | 31 | x5 | FF | FF | FF |

D:	 x ∈ {0x0; …; 0x9}

If the test/sequence is executed using an explicit test option as identified in Table A.2/1 or A.2/2, providing log data for all UICC-Terminal communication, x may be set to x=7.

### 4.6.3	EFIMSI "short", with 2-digit MNC and random value

EFIMSI (International Mobile Subscriber Identity)

Logically:

24681357x

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 05 | 29 | 64 | 18 | 53 | x7 | FF | FF | FF |

D:	 x ∈ {0x0; …; 0x9}

If the test/sequence is executed using an explicit test option as identified in Table A.2/1 or A.2/2, providing log data for all UICC-Terminal communication, x may be set to x=9.

### 4.6.4	EFIMSI "long", with 3-digit MNC and random value

EFIMSI (International Mobile Subscriber Identity)

Logically:

24608111111111x

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 08 | 29 | 64 | 80 | 11 | 11 | 11 | 11 | x1 |

D:	 x ∈ {0x0; …; 0x9}

If the test/sequence is executed using an explicit test option as identified in Table A.2/1 or A.2/2, providing log data for all UICC-Terminal communication, x may be set to x=1.

### 4.6.5	EFIMSI for SUCI calculation tests

EFIMSI (International Mobile Subscriber Identity)

Logically:

24608135793579x

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 08 | 29 | 64 | 80 | 31 | 75 | 39 | 75 | x9 |

D:	 x ∈ {0x0; …; 0x9}

If the test/sequence is executed using an explicit test option as identified in Table A.2/1 or A.2/2, providing log data for all UICC-Terminal communication, x may be set to x=3.

### 4.6.6	EFIMSI for "change IMSI" tests

EFIMSI (International Mobile Subscriber Identity)

Logically:

24608122223333x

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 08 | 29 | 64 | 80 | 21 | 22 | 32 | 33 | X3 |

D:	 x ∈ {0x0; …; 0x9}

If the test/sequence is executed using an explicit test option as identified in Table A.2/1 or A.2/2, providing log data for all UICC-Terminal communication, x may be set to x=3.

### 4.6.7	EFRouting_Indicator with 2-digit MNC and random value

EFRouting_Indicator (Routing Indicator EF)

Logically:

1y

Coding:

| Byte | B1 | B2 | B3 | B4 |
| --- | --- | --- | --- | --- |
| Hex | y1 | FF | FF | FF |

D:	 y ∈ {0x0; …; 0x9, 0xF}

If the test/sequence is executed using an explicit test option as identified in Table A.2/1 or A.2/2, providing log data for all UICC-Terminal communication, y may be set to y=7.

### 4.6.8	EFLOCI with "short" random TMSI value

EFLOCI (Location Information)

Logically:

LAI-MCC:		246

LAI-MNC:		081

LAI-LAC:		0001

TMSI:				0000214x

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 00 | 00 | 21 | 4x | 42 | 16 | 80 | 00 | 01 | FF | 00 |

D:	 x ∈ {0x0; …; 0x9}

If the test/sequence is executed using an explicit test option as identified in Table A.2/1 or A.2/2, providing log data for all UICC-Terminal communication, x may be set to x=3.

### 4.6.9	EFLOCI with "long" random TMSI value

EFLOCI (Location Information)

Logically:

LAI-MCC:	246

LAI-MNC:	081

LAI-LAC:	0001

TMSI:			214x0000

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 21 | 4x | 00 | 00 | 42 | 16 | 80 | 00 | 01 | FF | 00 |

D:	 x ∈ {0x0; …; 0x9}

If the test/sequence is executed using an explicit test option as identified in Table A.2/1 or A.2/2, providing log data for all UICC-Terminal communication, x may be set to x=3.

### 4.6.10	EFEPSLOCI

EFEPSLOCI (EPS Location Information)

Logically:

GUTI:								24608100010266uvwxyz

Last visited registered TAI:		246/081/0001

EPS update status:				not updated

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 0B | F6 | 42 | 16 | 80 | 00 | 01 | 02 |
|  | B9 | B10 | B11 | B12 | B13 | B14 | B15 | B16 |
|  | 66 | vu | xw | zy | 42 | 16 | 80 | 00 |
|  | B17 | B18 |
|  | 01 | 01 |

D:	u, v, w, x, y, z ∈ {0x0; …; 0x9}

If the test/sequence is executed using an explicit test option as identified in Table A.2/1 or A.2/2, providing log data for all UICC-Terminal communication, the variables may be set to fixed values: u=3, v=4, w=5, x=6, y=7, z=8

### 4.6.11	EFFDN

EFFDN (Fixed Dialling Numbers)

Logically:

Record 1:

Length of alpha identifier:	6 characters;

Alpha identifier:				"FDN111";

Length of BCD number:		6 bytes;

TON and NPI:				Telephony and International;

Dialled number:				+135792468x;

CCI2:							None;

Ext2: 							None.

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 46 | 44 | 4E | 31 | 31 | 31 | 06 | 91 |
|  | B9 | B10 | B11 | B12 | B13 | B14 | B15 | B16 |
|  | 31 | 75 | 29 | 64 | x8 | FF | FF | FF |
|  | B17 | B18 | B19 | B20 |
|  | FF | FF | FF | FF |

D:	x ∈ {0x0; …; 0x9}

Record 2:

Length of alpha identifier:	6 characters;

Alpha identifier:				"FDN222";

Length of BCD number:		4 bytes;

TON and NPI:				Telephony and International;

Dialled number:				2468x;

CCI2:							None;

Ext2: 							None.

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 46 | 44 | 4E | 32 | 32 | 32 | 04 | 81 |
|  | B9 | B10 | B11 | B12 | B13 | B14 | B15 | B16 |
|  | 42 | Fy | FF | FF | FF | FF | FF | FF |
|  | B17 | B18 | B19 | B20 |
|  | FF | FF | FF | FF |

D:	y ∈ {0x0; …; 0x9}

Record 3:

Length of alpha identifier:	6 characters;

Alpha identifier:				"FDN333";

Length of BCD number:		11 bytes;

TON and NPI:				Telephony and International;

Dialled number:				+1234567890123456789z;

CCI2:							None;

Ext2: 							None.

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 46 | 44 | 4E | 33 | 33 | 33 | 0B | 91 |
|  | B9 | B10 | B11 | B12 | B13 | B14 | B15 | B16 |
|  | 21 | 43 | 65 | 87 | 09 | 21 | 43 | 65 |
|  | B17 | B18 | B19 | B20 |
|  | 87 | z9 | FF | FF |

D:	z ∈ {0x0; …; 0x9}

If the test/sequence is executed using an explicit test option as identified in Table A.2/1 or A.2/2, providing log data for all UICC-Terminal communication, the variables may be set to fixed values: x, y, z = 0

### 4.6.12	EFECC

EFECC  (Emergency Call Codes)

Logically:

Emergency call code:					12x;

Emergency call code alpha identifier:	"TEST";

Emergency call Service Category:		Mountain Rescue.

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 21 | Fx | FF | 54 | 45 | 53 | 54 | 10 |

D:	x ∈ {0x0; …; 0x9}

If the test/sequence is executed using an explicit test option as identified in Table A.2/1 or A.2/2, providing log data for all UICC-Terminal communication, x may be set to x=2.

## 4.7	Default values for NG-SS and UE configurations

### 4.7.1	Default NG-SS and UE configuration for different usage settings

For 'voice centric' usage:

-	The NG-SS shall support IMS voice over PS session over 3GPP access and the IMS SIP Registration procedure, in accordance with clauses 4.7.2 and 4.7.3.

-	The UE shall use IMS specific configuration as defined in clause 4.7.4.

For 'data centric' usage:

-	The NG-SS shall support the default NG-SS configuration for IMS Registration as defined in clause 4.7.2.1. It may support IMS voice over PS session over 3GPP access as defined in clause 4.7.2.2.

-	The UE may attempt to initiate a PDU Session establishment for IMS and it is up to NG-SS implementation to accept it and to perform the IMS registration procedure as defined in clause 4.7.2 or to reject the PDU Session establishment. The IMS specific configuration as defined in clause 4.7.4 may be used in the UE.

In case the optional feature pc_VOICE_CENTRIC, as defined in table A.1 is supported, the TT shall operate the test case as defined for 'voice centric' usage. Else the 'data centric' configuration shall be used.

### 4.7.2	Default NG-SS configuration for 3GPP access with IMS service

#### 4.7.2.1	Default NG-SS configuration for IMS Registration support

The NG-SS is configured with the IMSI within the USIM application, the P-CSCF address, the home domain name, public and private user identities together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the ISIM.

-	NG-SS is listening to SIP default port 5060 for both UDP and TCP protocols.

-	NG-SS is able to perform IMS AKA authentication for the IMPI, according to TS 33.203 [57 clause 6.1.

-	SIB 1 in NG-SS Cell shall not include uac-BarringInfo with UAC_BarringInfo for AccessCategory 9, IMS Signalling.

#### 4.7.2.2	Default NG-SS configuration for PS voice support

The following parameters apply to all 5G test cases in 'voice centric' usage.

-	5GS network feature support element includes 'IMS voice over PS session over 3GPP access indicator' (IMS-VoPS-3GPP) in the REGISTRATION ACCEPT.

### 4.7.3	Default NG-SS procedure for 5G Registration on 3GPP access with IMS service

The NG-SS and UE shall perform 5G Registration procedure and IMS SIP Registration procedure as per the generic procedure defined in this clause.

5G-NR generic procedure for 5G Registration on 3GPP access with IMS service defined in Annex C.1 shall be the default procedure for 5G Registration on 3GPP access with IMS service.

The procedure provided for a test case may include common test steps only. However, in 'voice centric' usage, NG-SS and UE shall support additional steps defined in the 'voice centric' procedure in Annex C.1.

### 4.7.4	Default UE configuration for IMS service

UE shall be configured with the DNN type as 'ims' and DNN as IMS.

IMS private user identity, Home Network Domain Name, IMS public user identity, and P-CSCF Address required for IMS SIP Registration shall be configured in the 5G-NR ISIM-UICC as defined in clause 4.5.13.

# 5	Subscription related tests

## 5.1	IMSI/TMSI handling

### 5.1.1	UE identification by short IMSI

GERAN/UTRAN test - not applicable

### 5.1.2	UE identification by short IMSI using a 2-digit MNC

GERAN/UTRAN test - not applicable

### 5.1.3	UE identification by "short" TMSI

GERAN/UTRAN test - not applicable

### 5.1.4	UE identification by "long" TMSI

GERAN/UTRAN test - not applicable

### 5.1.5	UE identification by long IMSI, TMSI updating and key set identifier assignment

GERAN/UTRAN test - not applicable

### 5.1.6	UE identification by short IMSI when accessing E-UTRAN/EPC

#### 5.1.6.1	Definition and applicability

PAGING for EPS services using IMSI is an abnormal procedure used for error recovery in the network. The IMSI is used for unique identification of the UE by an E-UTRAN/EPC if there is no GUTI available. The IMSI is stored in the USIM and read during the UICC-Terminal initialisation procedure.

#### 5.1.6.2	Conformance requirement

CR 1	Only after reception of a PAGING message containing the IMSI stored in the USIM the UE shall send the RRC CONNECTION REQUEST message.

Reference:

-	TS 31.102 [19], clauses 5.1.1 and 5.2.2;

-	TS 24.301 [21], clause 5.6.2.2.2 and 5.6.2.4.

CR 2	For NB-IoT, the PAGING message shall include a CN domain indicator set to "PS". If the paging message includes a UE PAGING IDENTITY set to the UE’s IMSI, the PAGING procedure is performed according to clause 5.6.2.2.2 of [21].

Reference:

-	TS 31.102 [19], clauses 5.1.1 and 5.2.2;

-	TS 24.301 [21], clause 5.6.2.2.2 and 5.6.2.4.

CR 3	The ME correctly performs the READ BINARY command on EFIMSI.

Reference:

-	TS 31 101 [33], clause 11.1.3;

-	ETSI TS 102 221 [8], clause 11.1.3 and 14.1.1.

#### 5.1.6.3	Test purpose

The purpose of this test is to verify that:

1)	the ME uses the IMSI stored in the USIM when attaching to the network;

2)	the ME can handle an IMSI of less than the maximum length.

3)	To verify that the READ EFIMSI command is performed correctly by the ME.

4)	To verify that the UE does not respond to a paging message containing an IMSI not stored in the USIM.

#### 5.1.6.4	Method of test

##### 5.1.6.4.1	Initial conditions

The values of the E-UTRAN/EPC UICC as defined in clause 4.5.4 of the present document are used with EFIMSI as defined in clause 4.6.2.

For Test Procedure A the TT (E-USS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):		246/081/0001

-	Access control:				unrestricted

For Test Procedure B the TT (NB-SS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):		246/081/0001

-	Access control:				unrestricted

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and runs an initial activation.

##### 5.1.6.4.2	Procedure

###### 5.1.6.4.2.1	Test procedure A

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE > TT | Send ATTACH REQUEST |  |  |  |
| 2 | TT > UE | Send PAGING with IMSI 24608122222 | The UE does not send an RCC CONNECTION REQUEST |  |  |
| 3 | UE | READ EFIMSI |  | CR 3 | A.2/1 OR A.2/2 |
| 4 | TT > UE | Send PAGING with the IMSI stored in the USIM |  |  |  |
| 5 | UE > TT | Send RRC CONNECTION REQUEST | The TT responds with a RRC CONNECTION SETUP | CR 1 |  |
| 6 | UE > TT | Send RRC CONNECTION SETUP COMPLETE | The UE performs the EPS attach procedure |  |  |
| 7 | TT > UE | Send RRC CONNECTION RELEASE |  |  |  |

###### 5.1.6.4.2.2	Test procedure B

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE > TT | Send ATTACH REQUEST |  |  |  |
| 2 | TT > UE | Send PAGING-NB with IMSI 24608122222 | The UE does not send an RCC CONNECTION REQUEST-NB |  |  |
| 3 | UE | READ EFIMSI |  | CR 3 | A.2/1 OR A.2/2 |
| 4 | TT > UE | Send PAGING-NB with the IMSI stored in the USIM |  |  |  |
| 5 | UE > TT | Send RRC CONNECTION REQUEST-NB | The TT responds with a RRC CONNECTION SETUP-NB | CR 2 |  |
| 6 | UE > TT | Send RRC CONNECTION SETUP COMPLETE-NB | The UE performs the EPS attach procedure |  |  |
| 7 | TT > UE | Send RRC CONNECTION RELEASE-NB |  |  |  |

#### 5.1.6.5	Acceptance criteria

##### 5.1.6.5.1	Acceptance criteria for Test Procedure A

CR 1 is verified by analysing the IMSI value transferred by the UE in the RRC CONNECTION REQUEST. The conformance requirement CR 1 is met if the IMSI value stored on the USIM matches the IMSI value provided to the TT (USS).

##### 5.1.6.5.2	Acceptance criteria for Test Procedure B

CR 2 is verified by analysing the IMSI value transferred by the UE in the RRC CONNECTION REQUEST-NB. The conformance requirement CR 2 is met if the IMSI value stored on the USIM matches the IMSI value provided to the TT (NB-SS).

##### 5.1.6.4.3	Common acceptance criteria

CR 3 is explicitly verified in step 3) by analysing the READ command used to read the contents of EFIMSI (A.2/1 or A.2/2). CR 3 is met if the READ command is executed as defined in [28], clause 14.1.1.

### 5.1.7	UE identification by short IMSI using a 2-digit MNC when accessing E-UTRAN/EPC

#### 5.1.7.1	Definition and applicability

In some networks the IMSI identifying the E-UTRAN/EPC can consist of a 2-digit MNC. PAGING for EPS services using IMSI is an abnormal procedure used for error recovery in the network. The IMSI is used for unique identification of the UE by an E-UTRAN/EPC if there is no GUTI available. The IMSI is stored in the USIM and read during the UICC-Terminal initialisation procedure.

#### 5.1.7.2	Conformance requirement

CR 1	Only after reception of a PAGING message containing the IMSI stored in the USIM the UE shall send the RRC CONNECTION REQUEST message.

Reference:

-	TS 31.102 [19], clause 5.1.1 and 5.2.2;

-	ETSI TS 102 221 [28], clause 14.1.1;

-	TS 24.301 [21], clause 5.6.2.2.2 and 5.6.2.4.

CR 2	The ME correctly handles the an IMSI that contains a 2-digit MNC.

Reference:

-	TS 23 003 [34], clause 2.2;

#### 5.1.7.3	Test purpose

The purpose of this test is to verify that:

1)	the ME uses the IMSI stored in the USIM when attaching to the network;

2)	the ME is capable of handling an IMSI with a 2-digit MNC;

3)	the EFIMSI is correctly read by the ME;

4)	the ME does not respond to a PAGING message containing an IMSI not stored in the USIM.

#### 5.1.7.4	Method of test

##### 5.1.7.4.1	Initial conditions

The values of the E-UTRAN/EPC UICC as defined in clause 4.5.4 of the present document are used with EFIMSI as defined in clause 4.6.3, and the following exception:

EFAD (Administrative Data)

Logically:

UE operation mode:			normal operation

Additional information:		ciphering indicator feature disabled

MNC:							2 digits

Coding:

| Byte | B1 | B2 | B3 | B4 |
| --- | --- | --- | --- | --- |
| Hex | 00 | 00 | 00 | 02 |

For Test Procedure A the TT (E-USS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):		246/81/0001

-	Access control:				unrestricted

For Test Procedure B the TT (NB-SS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):		246/81/0001

-	Access control:				unrestricted

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and runs an initial activation.

##### 5.1.7.4.2	Procedure

###### 5.1.7.4.2.1	Test procedure A

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE > TT | Send ATTACH REQUEST |  |  |  |
| 2 | TT > UE | Send PAGING with IMSI 24608122222 | The UE does not send an RCC CONNECTION REQUEST |  |  |
| 3 | UE | READ EFIMSI |  |  |  |
| 4 | TT > UE | Send PAGING with the IMSI stored in the USIM |  |  |  |
| 5 | UE > TT | Send RRC CONNECTION REQUEST | The TT responds with a RRC CONNECTION SETUP | CR 1 CR 2 |  |
| 6 | UE > TT | Send RRC CONNECTION SETUP COMPLETE | The UE performs the EPS attach procedure |  |  |
| 7 | TT > UE | Send RRC CONNECTION RELEASE |  |  |  |

###### 5.1.7.4.2.2	Test procedure B

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE > TT | Send ATTACH REQUEST |  |  |  |
| 2 | TT > UE | Send PAGING-NB with IMSI 24608122222 | The UE does not send an RRCConnectionRequest-NB |  |  |
| 3 | UE | READ EFIMSI |  |  |  |
| 4 | TT > UE | Send PAGING-NB with the IMSI stored in the USIM |  |  |  |
| 5 | UE > TT | Send RRC CONNECTION REQUEST-NB | The TT responds with a RRC CONNECTION SETUP-NB | CR 1 CR 2 |  |
| 6 | UE > TT | Send RRC CONNECTION SETUP COMPLETE-NB | The UE performs the EPS attach procedure |  |  |
| 7 | TT > UE | Send RRC CONNECTION RELEASE-NB |  |  |  |

#### 5.1.7.4	Acceptance criteria

For Procedure A, CR 1 is verified by analysing the RRC CONNECTION REQUEST. The conformance requirement CR 1 is met if the IMSI value stored on the USIM matches the IMSI value provided to the TT (E-USS)

For Procedure B, CR 1 is verified by analysing the RRC CONNECTION REQUEST-NB. The conformance requirement CR 1 is met if the IMSI value stored on the USIM matches the IMSI value provided to the TT (NB-SS).

CR 2 can be implicitly verified by assuring that the RRC CONNECTION REQUEST/RRC CONNECTION REQUEST-NB sent from the UE contains the IMSI value stored on the USIM.

NOTE:	An explicit verification of the READ command is done in test 5.1.6 and another verification is not required. Nevertheless, it is optional to explicitly verify the READ EFIMSI command in step 2) using any of the test options A.2/1 or A.2/2.

### 5.1.8	UE identification after changed IMSI with service "EMM Information" not available

#### 5.1.8.1	Definition and applicability

The attach procedure is used to attach for packet services in EPS. With a successful attach procedure, a context is established for the UE in the MME, and a default bearer is established between the UE and the PDN GW, thus enabling always-on IP connectivity to the UE. The network may also initiate the activation of dedicated bearers as part of the attach procedure. For NB-IoT UEs the establishment of the PDN connection is optional.

#### 5.1.8.2	Conformance requirement

CR 1	The following EMM parameters shall be stored on the USIM if the corresponding file is present:

-	GUTI;

-	last visited registered TAI;

-	EPS update status.

Reference:

-	TS 31.102 [19], clause 5.1.1 and 5.2.2;

-	TS 24.301 [21], clause 5.5.1.2.1, 5.5.1.2.2, 5.5.1.2.4 and Annex C.

CR 2	The UE includes the IMSI stored in the USIM during the attach procedure.

#### 5.1.8.3	Test purpose

The purpose of this test is to verify that:

- the UE deletes existing EMM parameters from the UE's non-volatile memory in case a different IMSI is activated;

- the UE includes the IMSI stored in the USIM during the attach procedure.

#### 5.1.8.4	Method of test

##### 5.1.8.4.1	Initial conditions

The values of the Default UICC as defined in clause 4.5.2 of the present document are used with the following exception:

EFUST (USIM Service Table)

Logically:

| Service n°85: |  | EPS Mobility Management Information | not available |
| --- | --- | --- | --- |

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary | xx1x xx11 | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxxx xxxx | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | B12 |
|  | xxxx xxxx | xxxx xxxx | xx10 xxxx | 0000 0xxx |

The TT (E-USS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):		246/081/0001

-	Access control:				unrestricted

The TT (NB-SS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):		246/081/0001

-	Access control:				unrestricted

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and runs an initial activation.

##### 5.1.8.4.2	Procedure

###### 5.1.8.4.2.1	Test procedure A

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE > TT | Send RRC CONNECTION REQUEST | The TT responds with a RRC CONNECTION SETUP |  |  |
| 2 | UE > TT | Send RRC CONNECTION SETUP COMPLETE |  |  |  |
| 3 | UE > TT | Send ATTACH REQUEST | The TT responds with an ATTACH ACCEPT including the following values: - GUTI: "24608100010266345678" - TAI (MCC/MNC/TAC): 246/081/0001 |  |  |
| 4 | UE > TT | Send ATTACH COMPLETE | The TT responds with a RRC CONNECTION RELEASE |  |  |
| 5 | USER/TT | Power off/deactivate the UE |  |  |  |
| 6 | USER/TT | Reconfigure UICC in the UE | The values of the Default UICC as defined in clause 4.5.2 of the present document are used with EFIMSI as defined in clause 4.6.6 |  |  |
| 7 | USER/TT | Run activation of the UE |  |  |  |
| 8 | UE > TT | Send RRC CONNECTION REQUEST | The TT responds with a RRC CONNECTION SETUP |  |  |
| 9 | UE > TT | Send RRC CONNECTION SETUP COMPLETE |  |  |  |
| 10 | UE > TT | Send ATTACH REQUEST | The UE sends an ATTACH REQUEST including the IMSI as generated in accordance to the definition in clause 4.6.6 but neither a GUTI nor the TAI | CR 1 CR 2 |  |

###### 5.1.8.4.2.2	Test procedure B

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE > TT | Send RRC CONNECTION REQUEST-NB | The TT responds with a RRC CONNECTION SETUP-NB |  |  |
| 2 | UE > TT | Send RRC CONNECTION SETUP COMPLETE-NB |  |  |  |
| 3 | UE > TT | Send ATTACH REQUEST | The TT responds with an ATTACH ACCEPT including the following values: - GUTI: "24608100010266345678" - TAI (MCC/MNC/TAC): 246/081/0001 |  |  |
| 4 | UE > TT | Send ATTACH COMPLETE | The TT responds with a RRC CONNECTION RELEASE-NB |  |  |
| 5 | USER/TT | Power off/deactivate the UE |  |  |  |
| 6 | USER/TT | Reconfigure UICC in the UE | The values of the Default UICC as defined in clause 4.5.2 of the present document are used with EFIMSI as defined in clause 4.6.6 |  |  |
| 7 | USER/TT | Run activation of the UE |  |  |  |
| 8 | UE > TT | Send RRC CONNECTION REQUEST-NB | The TT responds with a RRC CONNECTION SETUP COMPLETE-NB |  |  |
| 9 | UE > TT | Send RRC CONNECTION SETUP COMPLETE-NB |  |  |  |
| 10 | UE > TT | Send ATTACH REQUEST | The UE sends an ATTACH REQUEST including the IMSI as generated in accordance to the definition in clause 4.5.84.6.6 but neither a GUTI nor the TAI | CR 1 CR 2 |  |

#### 5.1.8.5	Acceptance criteria

CR 1 and CR 2 can be implicitly verified by assuring that the ATTACH REQUEST sent from the UE in step 10) contains the IMSI value provided during reconfiguration (step 6)).

NOTE:	As service n°85 is not available, no EMM parameters shall be stored on the USIM. Further verification of CR 1 is not needed.

### 5.1.9	UE identification by GUTI when using USIM with service "EMM Information" not available

#### 5.1.9.1	Definition and applicability

The attach procedure is used to attach for packet services in EPS. With a successful attach procedure, a context is established for the UE in the MME, and a default bearer is established between the UE and the PDN GW, thus enabling always-on IP connectivity to the UE. The network may also initiate the activation of dedicated bearers as part of the attach procedure. For NB-IoT UEs the establishment of the PDN connection is optional.

#### 5.1.9.2	Conformance requirement

CR 1	The following EMM parameters shall be stored on the USIM if the corresponding file is present:

-	GUTI;

-	last visited registered TAI;

-	EPS update status

NOTE:	If the corresponding file is not present on the USIM, these EMM parameters except allowed CSG list are stored in a non-volatile memory in the ME together with the IMSI from the USIM. These EMM parameters can only be used if the IMSI from the USIM matches the IMSI stored in the non-volatile memory; else the UE shall delete the EMM parameters.

Reference:

-	TS 31.102 [19], clause 5.1.1 and 5.2.2;

-	TS 24.301 [21], clause 5.5.1.2.1, 5.5.1.2.2, 5.5.1.2.4 and Annex C.

CR 2	The UE uses the GUTI and the TAI from the UE's non-volatile memory during the attach procedure if the IMSI stored in the UE's non-volatile memory and the IMSI stored in the USIM are identical.

#### 5.1.9.3	Test purpose

The purpose of this test is to verify that:

1)	the UE stores the GUTI and the TAI in the UE's non-volatile memory;

2)	the UE includes the IMSI stored in the USIM during the attach procedure.

#### 5.1.9.4	Method of test

##### 5.1.9.4.1	Initial conditions

The values of the Default UICC as defined in clause 4.5.2 of the present document are used with the following exception:

EFUST (USIM Service Table)

Logically:

| Service n°85: |  | EPS Mobility Management Information | not available |
| --- | --- | --- | --- |

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary | xx1x xx11 | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxxx xxxx | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | B12 |
|  | xxxx xxxx | xxxx xxxx | xx10 xxxx | 0000 0xxx |

The TT (E-USS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):		246/081/0001

-	Access control:				unrestricted

The TT (NB-SS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):		246/081/0001

-	Access control:				unrestricted

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and runs an initial activation.

##### 5.1.9.4.2	Procedure

###### 5.1.9.4.2.1	Test procedure A

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE > TT | Send RRC CONNECTION REQUEST | The TT responds with a RRC CONNECTION SETUP |  |  |
| 2 | UE > TT | Send RRC CONNECTION SETUP COMPLETE |  |  |  |
| 3 | UE > TT | Send ATTACH REQUEST | The UE sends an ATTACH ACCEPT including the following values: - GUTI: "24608100010266345678" - TAI (MCC/MNC/TAC): 246/081/0001 |  |  |
| 4 | UE > TT | Send ATTACH COMPLETE | The TT responds with a RRC CONNECTION RELEASE |  |  |
| 5 | USER/TT | Power off/deactivate the UE, then run activation of the UE |  |  |  |
| 6 | UE > TT | Send RRC CONNECTION REQUEST | The TT responds with a RRC CONNECTION SETUP |  |  |
| 7 | UE > TT | Send RRC CONNECTION SETUP COMPLETE |  |  |  |
| 8 | UE > TT | Send ATTACH REQUEST | The sent ATTACH ACCEPT includes the following values: - GUTI: "24608100010266345678" - TAI (MCC/MNC/TAC): 246/081/0001 | CR 1 CR 2 |  |

###### 5.1.9.4.2.2	Test procedure B

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE > TT | Send RRC CONNECTION REQUEST-NB | The TT responds with a RRC CONNECTION SETUP-NB |  |  |
| 2 | UE > TT | Send RRC CONNECTION SETUP COMPLETE-NB |  |  |  |
| 3 | UE > TT | Send ATTACH REQUEST | The TT sends an ATTACH ACCEPT including the following values: - GUTI: "24608100010266345678" - TAI (MCC/MNC/TAC): 246/081/0001 |  |  |
| 4 | UE > TT | Send ATTACH COMPLETE |  |  |  |
| 5 | TT > UE | Send RRC CONNECTION RELEASE-NB |  |  |  |
| 6 | USER/TT | Power off/deactivate the UE, then run activation of the UE |  |  |  |
| 7 | UE > TT | Send RRC CONNECTION REQUEST-NB | The TT responds with a RRC CONNECTION SETUP-NB |  |  |
| 8 | UE > TT | Send RRC CONNECTION SETUP COMPLETE-NB |  |  |  |
| 9 | UE > TT | Send ATTACH REQUEST | The sent ATTACH ACCEPT includes the following values: - GUTI: "24608100010266345678" - TAI (MCC/MNC/TAC): 246/081/0001 | CR 1 CR 2 |  |

#### 5.1.9.5	Acceptance criteria

CR 1 and CR 2 can be implicitly verified by assuring that the ATTACH REQUEST sent from the UE in step 10) contains the GUTI and TAI values provided during the first attach procedure.

NOTE:	As service n°85 is not available, no EMM parameters shall be stored on the USIM. Further verification of CR 1 is not needed.

### 5.1.10	UE identification by GUTI when using USIM with service "EMM Information" available

#### 5.1.10.1	Definition and applicability

The attach procedure is used to attach for packet services in EPS. With a successful attach procedure, a context is established for the UE in the MME, and a default bearer is established between the UE and the PDN GW, thus enabling always-on IP connectivity to the UE. The network may also initiate the activation of dedicated bearers as part of the attach procedure. For NB-IoT UEs the establishment of the PDN connection is optional.

#### 5.1.10.2	Conformance requirement

CR 1	The following EMM parameters shall be stored on the USIM if the corresponding file is present:

-	GUTI;

-	last visited registered TAI;

-	EPS update status

NOTE:	The presence and format of corresponding files on the USIM is specified in TS 31.102 [19].

Reference:

-	TS 31.102 [19], clause 5.1.1 and 5.2.2;

-	TS 24.301 [21], clause 5.5.1.2.1, 5.5.1.2.2, 5.5.1.2.4 and Annex C.

CR 2	The UE uses the GUTI and the TAI stored in EFEPSLOCI on the USIM during the attach procedure.

#### 5.1.10.3	Test purpose

The purpose of this test is to verify that:

1)	the UE stores the GUTI and the TAI in in EFEPSLOCI if service n°85 is available in EFUST;

2)	the UE uses GUTI and the TAI values stored in EFEPSLOCI during the attach procedure.

#### 5.1.10.4	Method of test

##### 5.1.10.4.1	Initial conditions

The values of the E-UTRAN/EPC UICC as defined in clause 4.5.4 of the present document are used with EFEPSLOCI as defined in clause 4.6.10 and the following exception:

EFEPSNSC (EPS NAS Security Context)

Logically:

Key Set Identifier KSIASME:				'07' (no key available)

ASME Key (KSIASME):					32 byte key, any value

Uplink NAS count:						'00'

Downlink NAS count:					'01'

Identifiers of selected NAS integrity
		  and encryption algorithm:				'01'

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | … | B39 | B40 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | A0 | 34 | 80 | 01 | 07 | 81 | 20 | xx | … | xx | 82 |
|  | B41 | B42 | B43 | B44 | B45 | B46 | B47 | B48 | B49 | B50 | B51 |
|  | 04 | 00 | 00 | 00 | 00 | 83 | 04 | 00 | 00 | 00 | 01 |
|  | B52 | B53 | B54 |
|  | 84 | 01 | 01 |

The TT (E-USS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):		246/081/0002

-	Access control:				unrestricted

The TT (NB-SS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):		246/081/0002

-	Access control:				unrestricted

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and runs an initial activation.

##### 5.1.10.4.2	Procedure

###### 5.1.10.4.2.1	Test procedure A

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE > TT | Send RRC CONNECTION REQUEST | The TT responds with a RRC CONNECTION SETUP |  |  |
| 2 | UE > TT | Send RRC CONNECTION SETUP COMPLETE |  |  |  |
| 3 | UE > TT | Send ATTACH REQUEST | Receiving the ATTACH REQUEST including the values available from EFEPSLOCI as defined in clause 4.6.10, the TT sends an ATTACH ACCEPT including the following values: - GUTI: "24608100010266436587" - TAI (MCC/MNC/TAC): 246/081/0002 | CR 2 |  |
| 4 | UE > TT | Send ATTACH COMPLETE | The TT responds with a RRC CONNECTION RELEASE |  |  |

###### 5.1.10.4.2.2	Test procedure B

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE > TT | Send RRC CONNECTION REQUEST-NB | The TT responds with a RRC CONNECTION SETUP-NB |  |  |
| 2 | UE > TT | Send RRC CONNECTION SETUP COMPLETE_NB |  |  |  |
| 3 | UE > TT | Send ATTACH REQUEST | Receiving the ATTACH REQUEST including the values available from EFEPSLOCI as defined in clause 4.6.10, the TT sends an ATTACH ACCEPT including the following values: - GUTI: "24608100010266436587" - TAI (MCC/MNC/TAC): 246/081/0002 | CR 2 |  |
| 4 | UE > TT | Send ATTACH COMPLETE | The TT responds with a RRC CONNECTION RELEASE-NB |  |  |

###### 5.1.10.4.2.3	Common final Test Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 5 OR 5+x | UE | READ EFEPSLOCI | The TT is enabled to verify the contents of EFEPSLOCI either while executing step 4) or in an additional action that might be executed after ending the test | CR 1 | A.2/1 OR A.2/2 OR A.2/3 |

#### 5.1.10.5	Acceptance criteria

CR 2 can be implicitly verified by assuring that the ATTACH REQUEST sent from the UE in step 3) contains the GUTI and TAI values provided from the EFEPSLOCI stored on the USIM during configuration.

CR 1 shall be explicitly verified using any of the methods identified in clause 3.7.2 of the present document. CR 1 is met if EFEPSLOCI contains the following data:

EFEPSLOCI (EPS Location Information)

Logically:

GUTI:								24608100010266436587

Last visited registered TAI:		246/081/0002

EPS update status:				updated

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 0B | F6 | 42 | 16 | 80 | 00 | 01 | 02 |
|  | B9 | B10 | B11 | B12 | B13 | B14 | B15 | B16 |
|  | 66 | 34 | 56 | 78 | 42 | 16 | 80 | 00 |
|  | B17 | B18 |
|  | 02 | 00 |

## 5.2	Access Control handling

### 5.2.1	Access Control information handling

GERAN/UTRAN test - not applicable

### 5.2.2	Access Control information handling for E-UTRAN/EPC

#### 5.2.2.1	Definition and applicability

Access Control allows restriction of EPS bearer context activation access attempts. All user equipment (UE) are assigned to one out of ten randomly allocated classes, and optionally (for priority uses) also to one or more special categories.

An Access Class of the special categories is only valid in the HPLMN or HPLMN country. Otherwise, the randomly allocated class is used.

The classes are programmed on the USIM. The network controls which classes at any time may be barred.

Emergency call handling is FFS.

#### 5.2.2.2	Conformance requirement

CR 1	The ME shall read the access control value as part of the USIM-Terminal initialisation procedure and subsequently adopt this value.

Reference:

-	TS 31.102 [19], clause 5.1.1.

CR 2	If the UE is a member of at least one access class which corresponds to the permitted classes as signalled over the air interface, and the access class is applicable in the serving network, access attempts are allowed. Otherwise access attempts are not allowed.

Reference:

-	TS 22.011 [6], clauses 4.3 and 4.4,

-	TS 24.301 [26], clause 5.5.1.2.6,

NOTE:	All options are shown in figure 5-2 of [2] and are referenced to the tests.

#### 5.2.2.3	Test purpose

The purpose of this test is to verify that:

1)	the ME reads the access control value as part of the USIM-Terminal initialisation procedure, and subsequently adopts this value;

2)	the UE controls its network access in accordance with its access control class and the conditions imposed by the serving network.

The tests verify ME performance for the following:

Tests (a) and (b):	UE with access class 0 to 9,

Tests (c):			UE with access class 11 and 15 not in HPLMN, and
				UE with access class 12,13 and 14 not in HPLMN country;

Test (d) and (e):	UE with access class 11 and 15 not in HPLMN, and
						UE with access class 12,13 and 14 not in HPLMN country;

Test (g) and (h):		UE with access class 11 and 15 in HPLMN, and
					UE with access class 12,13 and 14 in HPLMN country.

Each of the above are tested against all relevant combinations of access control bits signalled by the network, as shown in table 5-2 of TS 31.121 [2].

#### 5.2.2.4	Method of test

##### 5.2.2.4.1	Initial conditions

The values of the Default UICC as defined in clause 4.5.2 of the present document are used with the exceptions given in TS 31.121 [2] clauses 5.2.2.4.1 and 5.2.2.4.2.

The TT (E-USS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):	MCC, MNC: see table 5-2 of [2], TAC="0001".

-	Access control:	see table 5-2 of [2].

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case with an IMSI and access control values as given in table 5-2 of [2] and runs an initial activation.

NOTE:	Depending on the initial value of the EFEPSLOCI, the UE may perform a location update. This shall be accepted by the E-USS.

##### 5.2.2.4.2	Coding details

The coding details defined in TS 31.121 [2] clause 5.2.2.4.2 apply.

##### 5.2.2.4.3	Procedure

Execute the test procedure as defined in TS 31.121 [2] clause 5.2.2.4.3.

#### 5.2.2.5	Acceptance criteria

CR 1 is implicitly verified after step a) if the ME has adopted the access control parameters as requested.

CR 2 is verified after step b) if the ME accesses the network, or makes no access attempt, in accordance with the Test Result entries given in table 5-2 of [2].

NOTE 1:	barred = yes, in these sub-sequences, the UE shall not establish a connection
barred = no, the UE shall establish a connection.

NOTE 2:	For conformance testing, to limit testing, in test (a), (b) and (c) it is only necessary that one of the access classes is tested. This access class may be chosen randomly.

### 5.2.3	Access Control information handling for NB-IoT

#### 5.2.3.1	Definition and applicability

Access Control allows restriction RRC connection establishment attempts. All user equipment (UE) are assigned to one out of ten randomly allocated classes, and optionally (for priority uses) also to one or more special categories.

An Access Class of the special categories is only valid in the HPLMN or HPLMN country. Otherwise, the randomly allocated class is used.

The classes are programmed on the USIM. The network controls which classes at any time may be barred.

#### 5.2.3.2	Conformance requirement

CR 1	The ME shall read the access control value as part of the USIM-Terminal initialisation procedure and subsequently adopt this value.

Reference:

-	TS 31.102 [19], clause 5.1.1.

CR 2	If the UE is a member of at least one access class which corresponds to the permitted classes as signalled over the air interface, and the access class is applicable in the serving network, access attempts are allowed. Otherwise access attempts are not allowed.

Reference:

-	TS 22.011 [6], clauses 4.3 and 4.4,

-	TS 24.301 [26], clause 5.5.1.2.6,

#### 5.2.3.3	Test purpose

The purpose of this test is to verify that:

1)	the ME reads the access control value as part of the USIM-Terminal initialisation procedure, and subsequently adopts this value;

2)	the UE controls its network access in accordance with its access control class and the conditions imposed by the serving network.

The tests verify ME performance for the following:

Tests (a) and (b):	UE with access class 0 to 9,

Tests (c):			UE with access class 11 and 15 not in HPLMN, and
				UE with access class 12,13 and 14 not in HPLMN country;

Test (d) and (e):	UE with access class 11 and 15 not in HPLMN, and
						UE with access class 12,13 and 14 not in HPLMN country.

Each of the above are tested against all relevant combinations of access control bits signalled by the network, as shown in table 5-3 of TS 31.121 [2].

#### 5.2.3.4	Method of test

##### 5.2.3.4.1	Initial conditions

The values of the Default UICC as defined in clause 4.5.2 of the present document are used with the exceptions given in TS 31.121 [2] clauses 5.2.3.4.1 and 5.2.3.4.2.

The TT (E-USS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):	MCC, MNC: see table 5-3 of [2], TAC="0001".

-	Access control:	see table 5-3 of [2].

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case with an IMSI and access control values as given in table 5-3 of [2] and runs an initial activation.

NOTE:	Depending on the initial value of the EFEPSLOCI, the UE may perform a location update. This shall be accepted by the TT (NB-SS).

##### 5.2.3.4.2	Coding details

The coding details defined in TS 31.121 [2] clause 5.2.3.4.2 apply.

##### 5.2.3.4.3	Procedure

Execute the test procedure as defined in TS 31.121 [2] clause 5.2.3.4.3.

#### 5.2.3.5	Acceptance criteria

CR 1 is implicitly verified after step a) if the ME has adopted the access control parameters as requested.

CR 2 is verified after step a) if the ME accesses the network, or makes no access attempt, in accordance with the Test Result entries given in table 5-3 of [2].

NOTE 1:	barred = yes, in these sub-sequences, the UE shall not establish a connection
barred = no, the UE shall establish a connection.

NOTE 2:	For conformance testing, to limit testing, in test (a), (b) and (c) it is only necessary that one of the access classes is tested. This access class may be chosen randomly.

## 5.3	Handling subscription identifier privacy for 5G

### 5.3.1	SUCI calculation by ME using null scheme

#### 5.3.1.1	Definition and applicability

If the operator's decision is that ME shall calculate the SUCI, the home network operator shall provision a list of the Protection Scheme Identifiers in the USIM that the operator allows. The list of Protection Scheme Identifiers in the USIM may contain one or more Protection Scheme Identifiers in the order of their priority. The ME shall read the SUCI calculation information from the USIM, including the SUPI, the Home Network Public Key, the Home Network Public Key Identifier, and the list of Protection Scheme Identifiers. The ME shall select the protection scheme from its supported schemes that has the highest priority in the list obtained from the USIM.

The ME shall calculate the SUCI using the null-scheme if the highest priority of the protection schemes listed in the USIM is the null-scheme.

#### 5.3.1.2	Conformance requirement

CR 1	The SUCI calculation procedure shall be performed by the ME if Service n°124 is available and Service n°125 is not available in EFUST.

Reference:

-	TS 31.102 [19], clauses 4.4.11.8 and 4.4.11.11

CR 2	The ME shall read the SUCI calculation information from the USIM, including the SUPI, the Home Network Public Key, the Home Network, Public Key Identifier, and the list of Protection Scheme Identifiers.

Reference:

-	TS 31.102 [19], clauses 4.4.11.8 and 4.4.11.11 and clauses 5.3.47 and 5.3.51.

CR 3	The ME shall calculate the SUCI including the SUCI calculation information procedure and the routing indicator procedure, using the null-scheme if highest priority of the protection schemes listed in the USIM is the null-scheme.

Reference:

-	TS 31.102 [19], clauses 5.3.47 and 5.3.51;

-	TS 33.501 [24], clause Annex C.

CR 4	The UE shall successfully authenticate and perform an initial registration on the network.

Reference:

-	TS 24.501 [25], clause 5.5.1.2.4.

#### 5.3.1.3	Test purpose

The purpose of this test is to verify that:

1)	the ME correctly reads EFSUCI_Calc_Info, EFRouting_indicator and EFIMSI from the USIM,

2)	the SUCI calculation procedure is performed by the ME if Service n°124 is available and Service n°125 is not available in EFUST,

3)	the ME selects the protection scheme that has the highest priority from the list obtained by the USIM,

4)	the ME is calculating the SUCI using the null-scheme*,

5)	the UE successfully registers to the network.

NOTE:	In the used configuration the null-scheme is the protection scheme with the highest priority listed in the USIM.

#### 5.3.1.4	Method of test

##### 5.3.1.4.1	Initial conditions

The values of the 5G-NR UICC as defined in clause 4.5.9 of the present document are used with EFIMSI as defined in clause 4.6.5, EFRouting_Indicator as defined in clause 4.6.7, and the following exception:

EFSUCI_Calc_Info (Subscription Concealed Identifier Calculation Information EF)

Logically:

Protection Scheme Identifier List data object:

Protection Scheme Identifier 1 – null-scheme

Key Index 1: 0

Protection Scheme Identifier 2 – ECIES scheme profile B

Key Index 2: 1

Protection Scheme Identifier 3 – ECIES scheme profile A

Key Index 3: 2

Home Network Public Key List data object:

Home Network Public Key 1 Identifier:27

Home Network Public Key 1:

04 72 DA 71 97 62 34 CE 83 3A 69 07 42 58 67 B8 2E 07 4D 44 EF 90 7D FB 4B 3E 21 C1 C2 25 6E

BC D1 5A 7D ED 52 FC BB 09 7A 4E D2 50 E0 36 C7 B9 C8 C7 00 4C 4E ED C4 F0 68 CD 7B F8 D3

F9 00 E3 B4

Home Network Public Key 2 Identifier: 30

Home Network Public Key 2:

5A 8D 38 86 48 20 19 7C 33 94 B9 26 13 B2 0B 91 63 3C BD 89 71 19 27 3B F8 E4 A6 F4 EE C0 A6

50

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | A0 | 06 | 00 | 00 | 02 | 01 | 01 | 02 |
|  | B9 | B10 | B11 | B12 | B13 | B14 | B15 | B16 |
|  | A1 | 6B | 80 | 01 | 1B | 81 | 41 | 04 |
|  | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 72 | DA | 71 | 97 | 62 | 34 | CE | 83 |
|  | B25 | B26 | B27 | B28 | B29 | B30 | B31 | B32 |
|  | 3A | 69 | 07 | 42 | 58 | 67 | B8 | 2E |
|  | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 |
|  | 07 | 4D | 44 | EF | 90 | 7D | FB | 4B |
|  | B41 | B42 | B43 | B44 | B45 | B46 | B47 | B48 |
|  | 3E | 21 | C1 | C2 | 25 | 6E | BC | D1 |
|  | B49 | B50 | B51 | B52 | B53 | B54 | B55 | B56 |
|  | 5A | 7D | ED | 52 | FC | BB | 09 | 7A |
|  | B57 | B58 | B59 | B60 | B61 | B62 | B63 | B64 |
|  | 4E | D2 | 50 | E0 | 36 | C7 | B9 | C8 |
|  | B65 | B66 | B67 | B68 | B69 | B70 | B71 | B72 |
|  | C7 | 00 | 4C | 4E | ED | C4 | F0 | 68 |
|  | B73 | B74 | B75 | B76 | B77 | B78 | B79 | B80 |
|  | CD | 7B | F8 | D3 | F9 | 00 | E3 | B4 |
|  | B81 | B82 | B83 | B84 | B85 | B86 | B87 | B88 |
|  | 80 | 01 | 1E | 81 | 20 | 5A | 8D | 38 |
|  | B89 | B90 | B91 | B92 | B93 | B94 | B95 | B96 |
|  | 86 | 48 | 20 | 19 | 7C | 33 | 94 | B9 |
|  | B97 | B98 | B99 | B100 | B101 | B102 | B103 | B104 |
|  | 26 | 13 | B2 | 0B | 91 | 63 | 3C | BD |
|  | B105 | B106 | B107 | B108 | B109 | B110 | B111 | B112 |
|  | 89 | 71 | 19 | 27 | 3B | F8 | E4 | A6 |
|  | B113 | B114 | B115 | B116 | B117 |
|  | F4 | EE | C0 | A6 | 50 |

The TT (NG-SS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):		244/083/000001

-	Access control:				unrestricted.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and runs an initial activation.

##### 5.3.1.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | READ EFUST | (Evaluation of service settings) | CR 1 |  |
| 2 | UE | READ EFIMSI, EFSUCI_Calc_Info, EFRouting_indicator |  | CR 2 | A.2/1 OR A.2/2 |
| 3 | ME | Perform SUCI calculation | The ME performs a SUCI calculation using null scheme | CR 3 |  |
| 4 | UE > TT | Send REGISTRATION REQUEST | The UE sends a REGISTRATION REQUEST with 5GS registration type IE as "initial registration" and 5GS mobile identity information element type "SUCI" |  |  |
| 5 | TT > UE | Send REGISTRATION ACCEPT | The TT sends a REGISTRATION ACCEPT with 5G-GUTI |  |  |
| 6 | UE > TT | Send REGISTRATION COMPLETE |  | CR 4 |  |

#### 5.3.1.5	Acceptance criteria

CR 1 and CR 3 are implicitly verified in step 4). The conformance requirements are met if the 5GS mobile identity IE in the REGISTRATION REQUEST performed in step 4) includes the following values:

-	SUPI format:									0

-	Home Network Identifier:					246/081

-	Routing Indicator:							1y

-	Protection Scheme Identifier:				00

-	Home Network Public Key Identifier:		0

-	Scheme output:								35793579x

CR 2 can be verified by a method explicitly verifying the correct execution of the READ commands on the listed EFs (A.2/1 or A.2/2). CR 2 is met if the READ procedure on EFSUCI_Calc_Info, EFRouting_indicator and EFIMSI has been executed as defined in [28], clause 14.1.1.

CR 4 is met if the UE sends REGISTRATION COMPLETE message to the TT (NG-SS) in step 6).

NOTE:	It is optional to explicitly verify CR 1 in step 1) using any of the test options A.2/1 or A.2/2, checking the service options set in EFUST.

### 5.3.2	SUCI calculation by ME using Profile B

#### 5.3.2.1	Definition and applicability

If the operator's decision is that ME shall calculate the SUCI, the home network operator shall provision a list of the Protection Scheme Identifiers that the operator allows in the USIM. The list of Protection Scheme Identifiers in the USIM may contain one or more Protection Scheme Identifiers in order of their priority. The ME shall read the SUCI calculation information from the USIM, including the SUPI, the Home Network Public Key, the Home Network Public Key Identifier, and the list of Protection Scheme Identifiers. The ME shall select the protection scheme from its supported schemes that has the highest priority in the list obtained from the USIM.

#### 5.3.2.2	Conformance requirement

CR 1	SUCI calculation procedure shall be performed by the ME if Service n°124 is available and Service n°125 is not available in EFUST

Reference:

-	TS 31.102 [19], clause 4.4.11.8

CR 2	As part of the SUCI calculation performed by the ME, the ME performs the reading procedure with EFSUCI_Calc_Info.

Reference:

-	TS 31.102 [19], clause 4.4.11.8 and 5.3.47

CR 3	The ME shall calculate the SUCI using the ECIES scheme profile B if highest priority of the protection schemes listed in the USIM is the ECIES scheme profile B.

Reference:

-	TS 31.102 [19], clause 4.4.11.8

-	TS 33.501 [24], Annex C, clause C.3 and C.4

-	TS 24.501 [25], clause 9.11.3

CR 4	The UE shall successfully authenticate and perform an initial registration on the network.

Reference:

-	TS 24.501 [25], clause 5.5.1.2.4.

#### 5.3.2.3	Test purpose

The purpose of this test is to verify that:

- the ME reads the SUCI calculation information from the USIM, including the SUPI, the Home Network Public Key, the Home Network Public Key Identifier, and the list of Protection Scheme Identifiers,

2)	the ME selects the protection scheme from its supported schemes that has the highest priority in the list obtained from the USIM (i.e. ECIES scheme profile B and the home network public key),

3)	the UE successfully registers to the network.

NOTE:	In the used configuration profile B is the protection scheme with the highest priority listed in the USIM.

#### 5.3.2.4	Method of test

##### 5.3.2.4.1	Initial conditions

The values of the default 5G-NR UICC defined in clause 4.5.9 of the present document are used with EFIMSI as defined in clause 4.6.5, EFRouting_Indicator as defined in clause 4.6.7.

The TT (NG-SS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):		244/083/000001.

-	Access control:				unrestricted.

The TT (NG-SS) shall be configured with Home Network Private Key as following:

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | F1 | AB | 10 | 74 | 47 | 7E | BC | C7 |
|  | B9 | B10 | B11 | B12 | B13 | B14 | B15 | B16 |
|  | 5F | 54 | EA | 1C | 5F | C3 | 68 | B1 |
|  | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 61 | 67 | 30 | 15 | 5E | 00 | 41 | AC |
|  | B25 | B26 | B27 | B28 | B29 | B30 | B31 | B32 |
|  | 44 | 7D | 63 | 01 | 97 | 5F | EC | DA |

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and runs an initial activation.

##### 5.3.2.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | READ EFUST | (Evaluation of service settings) | CR 1 |  |
| 2 | UE | READ EFSUCI_Calc_Info |  | CR 2 |  |
| 3 | ME | Perform SUCI calculation | The ME performs a SUCI calculation using Profile B | CR 3 |  |
| 4 | UE > TT | Send REGISTRATION REQUEST | The UE sends a REGISTRATION REQUEST with 5GS registration type IE as "initial registration" and 5GS mobile identity information element type "SUCI" |  |  |
| 5 | TT > UE | Send REGISTRATION ACCEPT | The TT sends a REGISTRATION ACCEPT with a new 5G-GUTI |  |  |
| 6 | UE > TT | Send REGISTRATION COMPLETE |  | CR 4 |  |

#### 5.3.2.5	Acceptance criteria

Actions required to fulfil CR 1, CR 2 and CR 3 are executed by the ME. The implicitly verification of these requirements is done in step 4). The conformance requirements are met if the 5GS mobile identity IE in the REGISTRATION REQUEST performed in step 4) includes the following values:

-	SUPI format:									0

-	Home Network Identifier:					246/081

-	Routing Indicator:							1y

-	Protection Scheme Identifier:				02

-	Home Network Public Key Identifier:		27

-	Scheme output:								ECC ephemeral public key, encryption of 35793579x and
												MAC tag value

CR 4 is met if the UE sends REGISTRATION COMPLETE message to the TT (NG-SS) in step 6).

NOTE:	It is optional to explicitly verify CR 1 and CR 2 using any of the test options A.2/1 or A.2/2, checking the READ commands on EFSUCI_Calc_Info (CR 2) and evaluating the service options settings in EFUST (CR 1).

### 5.3.3	UE identification by SUCI during initial registration – SUCI calculation by USIM using profile B

#### 5.3.3.1	Definition and applicability

If the operator's decision, indicated by the USIM, is that the USIM shall calculate the SUCI, then the USIM shall not give the ME any parameter for the calculation of the SUCI including the Home Network Public Key Identifier, the Home Network Public Key, and the Protection Scheme Identifier. If the ME determines that the calculation of the SUCI, indicated by the USIM, shall be performed by the USIM, the ME shall delete any previously received or locally cached parameters for the calculation of the SUCI including the Routing Indicator, the Home Network Public Key Identifier, the Home Network Public Key and the Protection Scheme Identifier.

#### 5.3.3.2	Conformance requirement

CR 1	SUCI calculation procedure shall be performed by the USIM if services n°124 and n°125 are available in EFUST.

Reference:

-	TS 31.102 [19], clause 4.4.11.8

CR 2	The ME shall use the GET IDENTITY command in SUCI context to retrieve the SUCI calculated by the USIM.

Reference:

-	TS 31.102 [19], clause 5.3.48

CR 3	This GET IDENTITY command shall be as per 7.5.2 in TS 31.102 [19].

Reference:

-	TS 31.102 [19], clause 4.4.11.8

-	TS 33.501 [24], Annex C, clause C.3 and C.4

-	TS 24.501 [25], clause 9.11.3

CR 4	The UE shall successfully authenticate and perform an initial registration on the network.

Reference:

-	TS 24.501 [25], clause 5.5.1.2.4.

#### 5.3.3.3	Test purpose

The purpose of this test is to verify that:

- the GET IDENTITY command is performed correctly by the ME;

2)	the ME includes the SUCI received from the 5G-NR UICC within GET IDENTITY response in the 5GS mobile identity IE

#### 5.3.3.4	Method of test

##### 5.3.3.4.1	Initial conditions

The values of the default 5G-NR UICC defined in clause 4.5.9 of the present document are used with EFIMSI as defined in clause 4.6.5 and the following exceptions:

EFUST (USIM Service Table)

Logically:

| Service n°125: |  | SUCI calculation by the USIM | available |
| --- | --- | --- | --- |

Coding:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary: | xxxx xx1x | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxxx xxxx | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | … | B16 |  |  |  |
|  | xxxx xxxx | xxxx xxxx | xx11 xxxx | ... | xxx1 111x |  |  |  |

5G-NR UICC is configured with:

Protection Scheme Identifier:				ECIES scheme profile B

Key Index:									1

Home Network Public Key Identifier:	27

Home Network Public Key:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 04 | 72 | DA | 71 | 97 | 62 | 34 | CE |
|  | B9 | B10 | B11 | B12 | B13 | B14 | B15 | B16 |
|  | 83 | 3A | 69 | 07 | 42 | 58 | 67 | B8 |
|  | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 2E | 07 | 4D | 44 | EF | 90 | 7D | FB |
|  | B25 | B26 | B27 | B28 | B29 | B30 | B31 | B32 |
|  | 4B | 3E | 21 | C1 | C2 | 25 | 6E | BC |
|  | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 |
|  | D1 | 5A | 7D | ED | 52 | FC | BB | 09 |
|  | B41 | B42 | B43 | B44 | B45 | B46 | B47 | B48 |
|  | 7A | 4E | D2 | 50 | E0 | 36 | C7 | B9 |
|  | B49 | B50 | B51 | B52 | B53 | B54 | B55 | B56 |
|  | C8 | C7 | 00 | 4C | 4E | ED | C4 | F0 |
|  | B57 | B58 | B59 | B60 | B61 | B62 | B63 | B64 |
|  | 68 | CD | 7B | F8 | D3 | F9 | 00 | E3 |
|  | B65 |
|  | B4 |

EFSUCI_Calc_Info:	Not available to the ME

The TT (NG-SS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):		244/083/000001.

-	Access control:				unrestricted.

The TT (NG-SS) shall be configured with Home Network Private Key for profile B:

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | F1 | AB | 10 | 74 | 47 | 7E | BC | C7 |
|  | B9 | B10 | B11 | B12 | B13 | B14 | B15 | B16 |
|  | 5F | 54 | EA | 1C | 5F | C3 | 68 | B1 |
|  | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 61 | 67 | 30 | 15 | 5E | 00 | 41 | AC |
|  | B25 | B26 | B27 | B28 | B29 | B30 | B31 | B32 |
|  | 44 | 7D | 63 | 01 | 97 | 5F | EC | DA |

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and runs an initial activation.

##### 5.3.3.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | READ EFUST | (Evaluation of service settings) |  |  |
| 2 | USIM | Perform SUCI calculation | The USIM performs a SUCI calculation using Profile B | CR 1 |  |
| 3 | ME > USIM | Send GET IDENTITY | The USIM returns the calculated SUCI. The GET IDENTITY command is sent with Identity Context in P2 as SUCI (0x01) | CR 2 CR3 | A.2/1 OR A.2/2 |
| 4 | UE > TT | Send REGISTRATION REQUEST | The UE sends a REGISTRATION REQUEST with 5GS registration type IE as "initial registration" and 5GS mobile identity information element type "SUCI" |  |  |
| 5 | TT > UE | Send REGISTRATION ACCEPT | The TT sends a REGISTRATION ACCEPT with a new 5G-GUTI |  |  |
| 6 | UE > TT | Send REGISTRATION COMPLETE |  | CR 4 |  |

#### 5.3.3.5	Acceptance criteria

CR 1 is implicitly verified. The requirement is met if the expected SUCI is included in the REGISTRATION REQUEST in step 4). The SUCI in the 5GS mobile identity IE. The REGISTRATION REQUEST shall include the following values:

-	SUPI format:									0

-	Home Network Identifier:					246/081

-	Routing Indicator:							1y

-	Protection Scheme Identifier:				02

-	Home Network Public Key Identifier:		27

-	Scheme output:								ECC ephemeral public key, encryption of 35793579x and
												MAC tag value

CR 2 and CR 3 can be verified by a method explicitly verifying the correct execution of the GET IDENTITY command using the expected values (A.2/1 or A.2/2).

CR 4 is met if the UE sends REGISTRATION COMPLETE message to the NG-SS in step 6).

### 5.3.4	UE identification by SUCI in response to IDENTITY REQUEST message

#### 5.3.4.1	Definition and applicability

The identification procedure is specified to request a particular UE to provide specific identification parameters, e.g. the SUCI or the IMEI. The SUCI is a privacy preserving identifier containing the concealed SUPI and IMEI is a format of PEI.

The network initiates the identification procedure by sending an IDENTITY REQUEST message to the UE and starting timer T3570. The IDENTITY REQUEST message specifies the requested identification parameters in the Identity type information element and the UE shall be ready to respond to an IDENTITY REQUEST message at any time whilst in 5GMM-CONNECTED mode.

#### 5.3.4.2	Conformance requirement

CR 1	A UE shall be ready to respond to an IDENTITY REQUEST message at any time whilst in 5GMM-CONNECTED mode.

CR 2	Upon receipt of the IDENTITY REQUEST message, if the Identity type IE in the IDENTITY REQUEST message is set to "SUCI", the UE shall:

-	if timer T3519 is not running, generate a fresh SUCI as specified in TS 33.501 [41], send an IDENTITY RESPONSE message with the SUCI, start timer T3519 and store the value of the SUCI sent in the IDENTITY RESPONSE message; and

-	if timer T3519 is running, send an IDENTITY RESPONSE message with the stored SUCI.

CR 3	If the REGISTRATION ACCEPT message contained a 5G-GUTI, the UE shall return a REGISTRATION COMPLETE message to the AMF to acknowledge the received 5G-GUTI, stop timer T3519 if running, and delete any stored SUCI.

Reference:

-	TS 31.102 [19], clauses 5.3.47 and 5.3.51;

-	TS 33.501 [24], clause Annex C.

CR 4	The ME shall correctly read EFSUCI_Calc_Info, EFRouting_indicator and EFIMSI from the USIM.

Reference:

-	TS 31.102 [19], clause 4.4.11.8 and 5.3.47

-	TS 24.501 [25], clause 5.5.1.2.4.

#### 5.3.4.3	Test purpose

The purpose of this test is to verify that:

- the ME correctly performs the READ commands for EFSUCI_Calc_Info, EFRouting_indicator and EFIMSI from the USIM,

- the UE will perform SUCI calculation procedure correctly;

- upon reception of the IDENTITY REQUEST message with Identity type IE set to "SUCI", the UE will:

-	if timer T3519 is not running, generate a fresh SUCI, send an IDENTITY RESPONSE message with the SUCI, start timer T3519 and store the value of the SUCI sent in the IDENTITY RESPONSE message; and

-	if timer T3519 is running, send an IDENTITY RESPONSE message with the stored SUCI;

- upon reception of the REGISTRATION ACCEPT message containing a 5G-GUTI UE deletes the stored SUCI and stops timer T3519 if running.

#### 5.3.4.4	Method of test

##### 5.3.4.4.1	Initial conditions

The values of the default 5G-NR UICC defined in clause 4.5.9 of the present document are used with EFIMSI as defined in clause 4.6.5, EFRouting_Indicator as defined in clause 4.6.7 and the following exception:

EF5GS3GPPLOCI (5GS 3GPP Location Information)

Logically:

5G-GUTI:						244 083 00010266436587

TAI (MCC/MNC/TAC):		244/083/000001

5GS update status:			5U2 NOT UPDATED

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 00 | 0B | F2 | 42 | 34 | 80 | 00 | 01 |
|  | B9 | B10 | B11 | B12 | B13 | B14 | B15 | B16 |
|  | 02 | 66 | 43 | 65 | 87 | 42 | 34 | 80 |
|  | B17 | B18 | B19 | B20 |
|  | 00 | 00 | 01 | 01 |

The TT (NG-SS) transmits on the BCCH, with the following network parameters:

Cell A:

-	TAI (MCC/MNC/TAC):		244/083/000001

-	Access control:				unrestricted.

Cell B:

-	TAI (MCC/MNC/TAC):		244/084/000001

-	Access control:				unrestricted.

The Cell A on the TT (NG-SS) is activated and transmits on the BCCH.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and runs an initial activation.

##### 5.3.4.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | READ EFIMSI, EFSUCI_Calc_Info, EFRouting_indicator |  | CR 4 | A.2/1 OR A.2/2 |
| 2 | UE > TT | Send REGISTRATION REQUEST | The UE sends a REGISTRATION REQUEST to Cell A with 5GS registration type IE as "initial registration" and 5GS mobile identity information element type "5G-GUTI" and starts the timer T3519 |  |  |
| 3 | TT > UE | Send IDENTITY REQUEST | The TT sends an IDENTITY REQUEST indicating that the Identity type information element is "SUCI" and starts the timer T3570 |  |  |
| 4 | UE > TT | Send IDENTITY RESPONSE | The UE sends an IDENTITY RESPONSE message with a fresh generated SUCI, starts the timer T3519 and stores the new SUCI | CR 1 CR 2 |  |
| 5 | TT > UE | Send IDENTITY REQUEST | The TT ignores the IDENTITY RESPONSE sent by the UE and re-sends the IDENTITY REQUEST indicating that the Identity type information element is "SUCI" before the expiry of T3519 |  |  |
| 6 | UE > TT | Send IDENTITY RESPONSE | The UE sends an IDENTITY RESPONSE message with the stored SUCI | CR 1 CR 2 |  |
| 7 | TT > UE | Send REGISTRATION ACCEPT | The TT sends a REGISTRATION ACCEPT message with a 5G-GUTI and stops timer T3570 (if running) |  |  |
| 8 | UE > TT | Send REGISTRATION COMPLETE | The UE stops the timers T3510 (if running) and T3519 and deletes the stored SUCI | CR 3 |  |
| 9 | TT | Deactivate Cell A on BCCH then activate Cell B on BCCH |  |  |  |
| 10 | UE > TT | Send REGISTRATION REQUEST | The UE sends a REGISTRATION REQUEST to Cell B with 5GS registration type IE as "mobility registration updating" or as "initial registration" and 5GS mobile identity information element type "5G-GUTI" and starts timer T3510 |  |  |
| 11 | TT > UE | Send IDENTITY REQUEST | The TT sends an IDENTITY REQUEST indicating that the Identity type information element is "SUCI" and starts timer T3570 |  |  |
| 12 | UE > TT | Send IDENTITY RESPONSE | The UE sends an IDENTITY RESPONSE message with a fresh generated SUCI, starts timer T3519 and stores the new SUCI | CR 1 CR 2 |  |
| 13 | TT > UE | Send REGISTRATION ACCEPT | The TT sends a REGISTRATION ACCEPT message with a 5G-GUTI and stops the timer T3570 (if running) |  |  |
| 14 | UE > TT | Send REGISTRATION COMPLETE | The UE sends a REGISTRATION COMPLETE message, stops the timers T3510 (if running) and T3519 and deletes the stored SUCI | CR 3 |  |

#### 5.3.4.5	Acceptance criteria

CR 1 and CR 2 are implicitly verified in step 4), step 6) and step 12). The conformance requirements are met if the UE sends an IDENTITY RESPONSE message with:

a fresh generated SUCI if timer T3519 is not running;

the stored SUCI if timer T3519 is running.

CR 3 is implicitly verified in step 8) and step 14). The conformance requirement is met if the UE sent a REGISTRATION COMPLETE message after receiving the 5G-GUTI.

CR 4 can be verified in step 1) by a method explicitly verifying the correct execution of the READ commands on the listed EFs (A.2/1 or A.2/2). CR 4 is met if the READ procedure on EFSUCI_Calc_Info, EFRouting_indicator and EFIMSI has been executed as defined in [28], clause 14.1.1.

### 5.3.5	UE identification by SUCI in response to IDENTITY REQUEST message with T3519 timer expiry

#### 5.3.5.1	Definition and applicability

The identification procedure is specified to request a particular UE to provide specific identification parameters, e.g. the SUCI or the IMEI. The SUCI is a privacy preserving identifier containing the concealed SUPI and IMEI is a format of PEI.

The network initiates the identification procedure by sending an IDENTITY REQUEST message to the UE and starting timer T3570. The IDENTITY REQUEST message specifies the requested identification parameters in the Identity type information element and the UE shall be ready to respond to an IDENTITY REQUEST message at any time whilst in 5GMM-CONNECTED mode.

#### 5.3.5.2	Conformance requirement

CR 1	A UE shall be ready to respond to an IDENTITY REQUEST message at any time whilst in 5GMM-CONNECTED mode.

CR 2	Upon receipt of the IDENTITY REQUEST message, if the Identity type IE in the IDENTITY REQUEST message is set to "SUCI", the UE shall:

-	if timer T3519 is not running, generate a fresh SUCI as specified in TS 33.501 [41], send an IDENTITY RESPONSE message with the SUCI, start timer T3519 and store the value of the SUCI sent in the IDENTITY RESPONSE message; and

-	if timer T3519 is running, send an IDENTITY RESPONSE message with the stored SUCI.

CR 3	If the REGISTRATION ACCEPT message contained a 5G-GUTI, the UE shall return a REGISTRATION COMPLETE message to the AMF to acknowledge the received 5G-GUTI, stop timer T3519 if running, and delete any stored SUCI.

Reference:

-	TS 31.102 [19], clauses 5.3.47 and 5.3.51;

-	TS 33.501 [24], clause Annex C.

CR 4	On expiry of timer T3519 (60s) the UE shall delete the stored SUCI

Reference:

-	TS 24.501 [25], Table 10.2.1.

CR 5	During initial registration the UE handles the 5GS mobile identity IE in the following order as defined in TS 24.501 [25] clause 5.5.1.2.2:

a)	a valid 5G-GUTI assigned by the same PLMN;

b)	a valid 5G-GUTI assigned by an equivalent PLMN;

c)	a valid 5G-GUTI assigned by any other PLMN;

d)	a SUCI is available in the UE.

#### 5.3.5.3	Test purpose

The purpose of this test is to verify that:

- the ME correctly performs the READ commands for EFSUCI_Calc_Info, EFRouting_indicator and EFIMSI from the USIM,

- upon reception of the IDENTITY REQUEST message with Identity type IE set to "SUCI", the UE will:

-	if timer T3519 is not running, generate a fresh SUCI, send an IDENTITY RESPONSE message with the SUCI, start timer T3519 and store the value of the SUCI sent in the IDENTITY RESPONSE message; and

-	if timer T3519 is running, send an IDENTITY RESPONSE message with the stored SUCI,

- upon expiry of T3519 UE deletes the stored SUCI,

- the UE handles the 5GS mobile identity IE in the correct order during initial registration and use 5G-GUTI as identity when it has both, a valid 5G-GUTI and the SUCI.

#### 5.3.5.4	Method of test

##### 5.3.5.4.1	Initial conditions

The values of the default 5G-NR UICC defined in clause 4.5.9 of the present document are used with EFIMSI as defined in clause 4.6.5, EFRouting_Indicator as defined in clause 4.6.7 and the following exception:

EF5GS3GPPLOCI (5GS 3GPP Location Information)

Logically:

5G-GUTI:							244 083 00010266436587

TAI (MCC/MNC/TAC):			244/083/000001

5GS update status:				5U2 NOT UPDATED

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 00 | 0B | F2 | 42 | 34 | 80 | 00 | 01 |
|  | B9 | B10 | B11 | B12 | B13 | B14 | B15 | B16 |
|  | 02 | 66 | 43 | 65 | 87 | 42 | 34 | 80 |
|  | B17 | B18 | B19 | B20 |
|  | 00 | 00 | 01 | 01 |

The TT (NG-SS) transmits on the BCCH, with the following network parameters:

Cell A:

-	TAI (MCC/MNC/TAC):		244/083/000001

-	CellIdentity:					000000001

-	Access control:				unrestricted.

Cell B :

-	TAI (MCC/MNC/TAC):		244/084/000001

-	CellIdentity:					000000002

-	Access control:				unrestricted.

The Cell A on the TT (NG-SS) is activated and transmits on the BCCH.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and runs an initial activation.

##### 5.3.5.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE > TT | Send REGISTRATION REQUEST | The UE sends a REGISTRATION REQUEST to Cell A with 5GS registration type IE as "initial registration" and 5GS mobile identity information element type "5G-GUTI" and starts timer T3510 | CR 5 |  |
| 2 | TT > UE | Send IDENTITY REQUEST | The TT sends an IDENTITY REQUEST message indicating that the Identity type information element is "SUCI" | CR 1 CR 2 |  |
| 3 | UE > TT | Send IDENTITY RESPONSE | The UE sends an IDENTITY RESPONSE message with a fresh generated SUCI, starts the timer T3519 and stores the new SUCI |  |  |
| 4 | TT | Deactivate Cell A on BCCH and then activate Cell B on BCCH | Execute before the expiry of timer T3519 and the UE stops timer T3510 |  |  |
| 5 | UE > TT | Send REGISTRATION REQUEST | While timer T3519 is running the UE sends a REGISTRATION REQUEST to Cell B with 5GS registration type IE as "initial registration" and 5GS mobile identity information element type "5G-GUTI" and starts timer T3510 | CR 5 |  |
| 6 | TT > UE | Send IDENTITY REQUEST | The TT sends an IDENTITY REQUEST indicating that the Identity type information element is "SUCI" |  |  |
| 7 | UE > TT | Send IDENTITY RESPONSE | The UE sends an IDENTITY RESPONSE message with the stored SUCI |  |  |
| 8 | TT | Deactivate Cell B on BCCH and thenactivate Cell A on BCCH | Execute after 70 sec (after timer T3519 expires) and the UE stops timer T3510 |  |  |
| 9 | UE > TT | Send REGISTRATION REQUEST | The UE sends a REGISTRATION REQUEST to Cell A with 5GS registration type IE as "initial registration" and 5GS mobile identity information element type "5G-GUTI" and starts timer T3510 | CR 5 |  |
| 10 | TT > UE | Send IDENTITY REQUEST | The TT sends an IDENTITY REQUEST indicating that the Identity type information element is "SUCI" |  |  |
| 11 | UE > TT | Send IDENTITY RESPONSE | The UE sends an IDENTITY RESPONSE message with the freshly generated SUCI, starts timer T3519 and stores the new SUCI | CR 1 CR 2 |  |
| 12 | TT > UE | Send REGISTRATION ACCEPT | The TT sends a REGISTRATION ACCEPT message with a 5G-GUTI |  |  |
| 13 | UE > TT | Send REGISTRATION COMPLETE | The UE sends the REGISTRATION COMPLETE message, stops timers T3510 and T3519 (if running) and deletes the stored SUCI | CR 3 CR 4 |  |

#### 5.3.5.5	Acceptance criteria

CR 1 and CR 2 are implicitly verified in step 2) and step 11). The conformance requirements are met if the UE sends an IDENTITY RESPONSE message with:

a fresh generated SUCI if timer T3519 is not running;

the stored SUCI if timer T3519 is running.

CR 3 is implicitly verified in step 13). The conformance requirement is met if the UE sent a REGISTRATION COMPLETE message after receiving the 5G-GUTI.

CR 4 is verified in step 13). The conformance requirement is met if no SUCI is stored on the UE, i.e., no two successive GET IDENTITY commands in steps 3), 7) and 11) are identical.

CR 5 is implicitly verified in step 1), step 5) and step 9). The conformance requirement is met if the expected 5GS mobile identity IE is sent in the REGISTRATION REQUEST during initial registration.

### 5.3.6	UE identification by SUCI in response to IDENTITY REQUEST message and AUTHENTICATION REJECT

#### 5.3.6.1	Definition and applicability

The identification procedure is specified to request a particular UE to provide specific identification parameters, e.g. the SUCI or the IMEI. The SUCI is a privacy preserving identifier containing the concealed SUPI and IMEI is a format of PEI.

The network initiates the identification procedure by sending an IDENTITY REQUEST message to the UE and starting timer T3570. The IDENTITY REQUEST message specifies the requested identification parameters in the Identity type information element and the UE shall be ready to respond to an IDENTITY REQUEST message at any time whilst in 5GMM-CONNECTED mode.

#### 5.3.6.2	Conformance requirement

CR 1	A UE shall be ready to respond to an IDENTITY REQUEST message at any time whilst in 5GMM-CONNECTED mode.

Reference:

-	TS 24.501 [25], clause 5.4.3.3

-	TS 31.102 [19], clauses 4.4.11.8, 4.4.11.11, 5.3.47 and 5.3.51

CR 2	Upon receipt of the IDENTITY REQUEST message, if the Identity type IE in the IDENTITY REQUEST message is set to "SUCI", the UE shall:

- if timer T3519 is not running, generate a fresh SUCI as specified in TS 33.501 [24], send an IDENTITY RESPONSE message with the SUCI, start timer T3519 and store the value of the SUCI sent in the IDENTITY RESPONSE message;

- and

- if timer T3519 is running, send an IDENTITY RESPONSE message with the stored SUCI. The ME shall use the GET IDENTITY command in SUCI context to retrieve the SUCI calculated by the USIM.

Reference:

-	TS 24.501 [25], clause 5.4.3.3

CR 3	If the REGISTRATION ACCEPT message contained a 5G-GUTI, the UE shall return a REGISTRATION COMPLETE message to the AMF to acknowledge the received 5G-GUTI, stop timer T3519 if running, and delete any stored SUCI

Reference:

-	TS 24.501 [25], clause 5.5.1.2.4

CR 4	If the AUTHENTICATION REJECT message is received by the UE, the UE shall abort any 5GMM signalling procedure, stop any of the timers T3510, T3516, T3517, T3519 or T3521 (if they were running), delete stored SUCI and enter state 5GMM-DEREGISTERED.

Reference:

-	TS 24.501 [25], clause 8.2.5

#### 5.3.6.3	Test purpose

The purpose of this test is to verify that:

- the READ EFSUCI_Calc_Info, EFRouting_Indicator and EFIMSI commands are performed correctly by the ME;

- the ME will perform SUCI calculation procedure correctly;

- upon reception of the IDENTITY REQUEST message with Identity type IE set to "SUCI", the UE will:

- if timer T3519 is not running, generate a fresh SUCI as specified in TS 33.501 [24], send an IDENTITY RESPONSE message with the SUCI, start timer T3519 and store the value of the SUCI sent in the IDENTITY RESPONSE message;

- and

- if timer T3519 is running, send an IDENTITY RESPONSE message with the stored SUCI;

- upon receiving the AUTHENTICATION REJECT the UE deletes the stored SUCI.

#### 5.3.6.4	Method of test

##### 5.3.6.4.1	Initial conditions

The values of the default 5G-NR UICC defined in clause 4.5.9 of the present document are used with EFIMSI as defined in clause 4.6.5 and the following exceptions:

EF5GS3GPPLOCI (5GS 3GPP Location Information)

Logically:

5G-GUTI:						244 083 00010266436587

TAI (MCC/MNC/TAC):		244/083/000001

5GS update status:			5U2 NOT UPDATED

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 00 | 0B | F2 | 42 | 34 | 80 | 00 | 01 |
|  | B9 | B10 | B11 | B12 | B13 | B14 | B15 | B16 |
|  | 02 | 66 | 43 | 65 | 87 | 42 | 34 | 80 |
|  | B17 | B18 | B19 | B20 |
|  | 00 | 00 | 01 | 01 |

The TT (NG-SS) transmits on the BCCH, with the following network parameters:

Cell A:

-	TAI (MCC/MNC/TAC):		244/083/000001

-	Access control:				unrestricted.

Cell B:

-	TAI (MCC/MNC/TAC):		244/084/000001

-	Access control:				unrestricted.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and runs an initial activation.

##### 5.3.6.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | TT | Bring up Cell A |  |  |  |
| 2 | UE > TT | Send REGISTRATION REQUEST | The UE sends a REGISTRATION REQUEST to Cell A with 5GS registration type IE as "initial registration" and 5GS mobile identity information element type "5G-GUTI" and starts the timer T3519 |  |  |
| 3 | TT > UE | Send IDENTITY REQUEST | The IDENTITY REQUEST is indicating that the Identity type information element is "SUCI" |  |  |
| 4 | UE > TT | Send IDENTITY RESPONSE | The UE sends an IDENTITY RESPONSE containing the fresh generated SUCI and starts the timer T3519 | CR 1 CR 2 CR 3 |  |
| 5 | TT > UE | Send AUTHENTICATION REQUEST |  |  |  |
| 6 | UE > TT | Send AUTHENTICATION RESPONSE |  |  |  |
| 7 | TT > UE | Send AUTHENTICATION REJECT |  |  |  |
| 8 | UE | Stop timer T3510 Stop timer T3519 Delete stored SUCI Abort any 5GMM signalling procedure | Enter state 5GMM-DEREGISTERED | CR 4 |  |
| 9 | TT | Bring down Cell A Bring up Cell B |  |  |  |
| 10 | USER/TT | Power off/deactivate the UE, then run activation of the UE |  |  |  |
| 11 | UE > TT | Send REGISTRATION REQUEST | The UE sends a REGISTRATION REQUEST to Cell B with 5GS registration type IE as "initial registration" and 5GS mobile identity information element type "SUCI" containing the fresh generated SUCI, starts the timers T3510 and T3519 and delete the stored SUCI |  |  |
| 12 | TT > UE | Send REGISTRATION ACCEPT | The REGISTRATION ACCEPT message contains a 5G-GUTI |  |  |
| 13 | UE > TT | Send REGISTRATION COMPLETE | The UE i.a. acknowledges the received 5G-GUTI, stops the timers T3510 and T3519 if running and delete the stored SUCI |  |  |
| NOTE: a verification of timer states as indicated in steps 2), 4), 11) and 13) is not required. An explicit verification may be possible on MEs providing the related information. |  |  |  |  |  |

#### 5.3.6.5	Acceptance criteria

CR 1, CR 2 and CR 3 are implicitly verified if the UE returns an IDENTITY RESPONSE containing the expected SUCI in step 4).

CR 4 can be verified on the TT. The requirement is met if no signalling is received from the UE during step 8).

### 5.3.7	SUCI calculation by the ME using null scheme – missing parameters for subscription identifier privacy support by the USIM

#### 5.3.7.1	Definition and applicability

If the operator's decision is that ME shall calculate the SUCI, the home network operator shall provision a list of the Protection Scheme Identifiers that the operator allows in the USIM. The list of Protection Scheme Identifiers in the USIM may contain one or more Protection Scheme Identifiers in order of their priority. The ME shall read the SUCI calculation information from the USIM, including the SUPI, the Home Network Public Key, the Home Network Public Key Identifier, and the list of Protection Scheme Identifiers. The ME shall select the protection scheme from its supported schemes that has the highest priority in the list obtained from the USIM.

The ME shall calculate the SUCI using the null-scheme if one or more parameters (i.e. Home Network Public Key, Protection Scheme Identifier) required for the calculation of the SUCI are not provisioned in the USIM.

#### 5.3.7.2	Conformance requirement

CR 1	SUCI calculation procedure shall be performed by the ME if Service n°124 is available and Service n°125 is "not available" in EFUST

Reference:

-	TS 31.102 [19], clause 5.3.47

CR 2	As part of the SUCI calculation performed by the ME, the ME shall perform reading procedure on EFSUCI_Calc_Info and EFRouting_Indicator.

Reference:

-	TS 31.102 [19], clauses 4.4.11.8, 4.4.11.11, 5.3.47 and 5.3.51

CR 3	The ME shall calculate the SUCI using the null-scheme if no Protection Scheme Identifier is provisioned in the USIM or if there is no Home Network Public Key configured in the USIM for the highest priority protection scheme configured in the USIM that the ME supports

Reference:

-	TS 33.501 [24], Annex C

#### 5.3.7.3	Test purpose

The purpose of this test is to verify that:

- the READ EFSUCI_Calc_Info, EFRouting_Indicator and EFIMSI commands are performed correctly by the ME;

2)	the ME correctly performs the SUCI calculation procedure using null-scheme.

#### 5.3.7.4	Method of test

##### 5.3.7.4.1	Initial conditions

The values of the default 5G-NR UICC defined in clause 4.5.9 of the present document are used with EFIMSI as defined in clause 4.6.5 and the following exceptions:

The USIM does not have the Home Network Public Key configured for the highest priority protection scheme configured in the USIM that the ME supports.

EFSUCI_Calc_Info (Subscription Concealed Identifier Calculation Information EF)

Logically:

null

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 |
| --- | --- | --- | --- | --- | --- | --- |
| Hex | A0 | 02 | 01 | 01 | A1 | 00 |

The TT (NG-SS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):		244/083/000001,

-	Access control:				unrestricted.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and runs an initial activation.

##### 5.3.7.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | READ EFIMSI, EFUST, EFSUCI_Calc_Info, EFRouting_indicator | (Evaluation of service settings) | CR 1 | A.2/1 OR A.2/2 |
| 2 | UE > TT | Send REGISTRATION REQUEST | The UE sends a REGISTRATION REQUEST to Cell A with 5GS registration type IE as "initial registration" and 5GS mobile identity information element type "SUCI" | CR 2 |  |
| 3 | TT > UE | Send REGISTRATION ACCEPT | The REGISTRATION ACCEPT message contains a 5G-GUTI |  |  |
| 4 | UE > TT | Send REGISTRATION COMPLETE | i.a. acknowledge the received 5G-GUTI |  |  |

#### 5.3.7.5	Acceptance criteria

CR 1 can be verified by a method explicitly verifying the correct execution of the READ commands on the listed EFs (A.2/1 or A.2/2).

CR 2 is met if the 5GS mobile identity IE in the REGISTRATION REQUEST contains the SUCI as coded below:

SUPI format:						0

Home Network Identifier:		246/081

Routing indicator:				17

Protection scheme id:			00

Home network public key Id:	0

Scheme output:					357935793

### 5.3.8	UE identification by 5G-GUTI – Last Registered TAI stored on USIM

#### 5.3.8.1	Definition and applicability

A globally unique temporary user identity for 5GS-based services, the 5G globally unique temporary identity (5G-GUTI), is used for identification within the signalling procedures. A UE supporting N1 mode includes a valid 5G-GUTI, if any is available, in the REGISTRATION REQUEST and DEREGISTRATION REQUEST messages.

#### 5.3.8.2	Conformance requirement

CR 1	The following 5GMM parameters shall be stored on the USIM if the corresponding file is present:

-	5G-Globally Unique Temporary Identifier (5G-GUTI);

-	Last visited registered Tracking Area Identity in 5GS (TAI);

-	5GS update status.

Reference:

-	TS 31.102 [19], clauses 4.4.11.2

CR 2	If the corresponding file is not present on the USIM, these 5GMM parameters are stored in a non-volatile memory in the ME together with the SUPI from the USIM. These 5GMM parameters can only be used if the SUPI from the USIM matches the SUPI stored in the non-volatile memory; else the UE shall delete the 5GMM parameters.

Reference:

-	TS 24.501 [25], clauses 5.3.3, 5.5.1.2 and Annex C.

#### 5.3.8.3	Test purpose

The purpose of this test is to verify that:

- the READ EF5GS3GPPLOCI and EFIMSI commands are performed correctly by the ME;

- the ME correctly uses the 5G-GUTI in the REGISTRATION REQUEST.

#### 5.3.8.4	Method of test

##### 5.3.8.4.1	Initial conditions

The values of the default 5G-NR UICC defined in clause 4.5.9 of the present document are used with EFIMSI as defined in clause 4.6.5 and the following exceptions:

EF5GS3GPPLOCI

Logically:

5G-GUTI:						244 083 00010266436587

TAI (MCC/MNC/TAC):		244/083/000001

5GS update status:			5U2 NOT UPDATED

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 00 | 0B | F2 | 42 | 34 | 80 | 00 | 01 |
|  | B9 | B10 | B11 | B12 | B13 | B14 | B15 | B16 |
|  | 02 | 66 | 43 | 65 | 87 | 42 | 34 | 80 |
|  | B17 | B18 | B19 | B20 |
|  | 00 | 00 | 01 | 01 |

The TT (NG-SS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):		244/083/000002

-	Access control:				unrestricted.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and runs an initial activation.

##### 5.3.8.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | TT | Bring up the NG-SS |  |  |  |
| 2 | UE | READ EFIMSI, EF5GS3GPPLOCI |  | CR 1 | A.2/1 OR A.2/2 |
| 3 | UE > TT | Send REGISTRATION REQUEST | The UE sends a REGISTRATION REQUEST to the NG-SS with 5GS registration type IE as "initial registration" and 5GS mobile identity information element type "5G-GUTI" |  |  |
| 4 | TT > UE | Send REGISTRATION ACCEPT | The REGISTRATION ACCEPT message contains: - 5G-GUTI: "24408300010266555555" - 5GS TAI list with TAI with: - TAI: 244/083/000002 |  |  |
| 5 | UE > TT | Send REGISTRATION COMPLETE | The UE i.a. acknowledges the received 5G-GUTI, sends the REGISTRATION COMPLETE message and stops timers T3510 (if running) |  |  |
| 6 | USER/TT | Power off/deactivate the UE, then run activation of the UE | The UE updates the USIM with valid NAS security context |  |  |
| 7 | UE | READ EFIMSI, EF5GS3GPPLOCI |  | CR 1 | A.2/1 OR A.2/2 |
| 8 | UE > TT | Send REGISTRATION REQUEST | The UE sends a REGISTRATION REQUEST to the NG-SS with 5GS registration type IE as "initial registration" and 5GS mobile identity information element type "5G-GUTI" and starts the timer T3510 | CR 2 |  |
| 9 | TT > UE | Send REGISTRATION ACCEPT | The REGISTRATION ACCEPT message contains a 5G-GUTI. The UE i.a. acknowledge the received 5G-GUTI |  |  |
| NOTE: a verification of timer states as indicated in steps 5) and 8) is not required. An explicit verification may be possible on MEs providing the related information. |  |  |  |  |  |

#### 5.3.8.5	Acceptance criteria

CR 1 can be verified by a method explicitly verifying the correct execution of the READ commands in steps 2) and 7) on the listed EFs (A.2/1 or A.2/2).

CR 2 is implicitly verified if the REGISTRATION REQUEST in step 7) contains the new 5G-GUTI and the last visited TAI:

SUPI format:					244 083 00010266555555

TAI (MCC/MNC/TAC):		244/083/000002

### 5.3.9	UE identification by 5G-GUTI – Last Registered TAI stored by ME

#### 5.3.9.1	Definition and applicability

A globally unique temporary user identity for 5GS-based services, the 5G globally unique temporary identity (5G-GUTI), is used for identification within the signalling procedures. A UE supporting N1 mode includes a valid 5G-GUTI, if any is available, in the REGISTRATION REQUEST and DEREGISTRATION REQUEST messages.

#### 5.3.9.2	Conformance requirement

CR 1	The following 5GMM parameters shall be stored on the USIM if the corresponding file is present:

-	5G-Globally Unique Temporary Identifier (5G-GUTI);

-	Last visited registered Tracking Area Identity in 5GS (TAI);

-	5GS update status.

Reference:

-	TS 31.102 [19], clauses 4.4.11.2

CR 2	If the corresponding file is not present on the USIM, these 5GMM parameters are stored in a non-volatile memory in the ME together with the SUPI from the USIM. These 5GMM parameters can only be used if the SUPI from the USIM matches the SUPI stored in the non-volatile memory; else the UE shall delete the 5GMM parameters.

Reference:

-	TS 24.501 [25], clauses 5.3.3, 5.5.1.2 and Annex C.

#### 5.3.9.3	Test purpose

The purpose of this test is to verify that:

- the READ EFIMSI commands are performed correctly by the ME;

- the ME correctly uses the 5G-GUTI in the REGISTRATION REQUEST;

- the ME stores the new 5G-GUTI in its non-volatile memory if the corresponding file is not present in the USIM.

#### 5.3.9.4	Method of test

##### 5.3.9.4.1	Initial conditions

The values of the default 5G-NR UICC defined in clause 4.5.9 of the present document are used with EFIMSI as defined in clause 4.6.5.

The TT (NG-SS) transmits on the BCCH, with the following network parameters:

Cell A:

-	TAI (MCC/MNC/TAC):		244/083/000001

-	Access control:				unrestricted.

Cell B:

-	TAI (MCC/MNC/TAC):		244/084/000001

-	Access control:				unrestricted.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and runs an initial activation.

##### 5.3.9.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | TT | Bring up the Cell A |  |  |  |
| 2 | UE | READ EFIMSI |  | CR 1 | A.2/1 OR A.2/2 |
| 3 | UE > TT | Send REGISTRATION REQUEST | The UE sends a REGISTRATION REQUEST to the NG-SS with 5GS registration type IE as "initial registration" |  |  |
| 4 | TT > UE | Send REGISTRATION ACCEPT | The REGISTRATION ACCEPT message contains: - 5G-GUTI: "24408300010266436587" - 5GS TAI list with TAI with: - TAI: 244/083/000002 |  |  |
| 5 | UE > TT | Send REGISTRATION COMPLETE |  |  |  |
| 6 | USER/TT | Power off/deactivate the UE, then run activation of the UE |  |  |  |
| 7 | UE > TT | Send REGISTRATION REQUEST | The UE sends a REGISTRATION REQUEST to the NG-SS with 5GS registration type IE as "initial registration" and 5GS mobile identity information element type "5G-GUTI" and starts the timer T3510 | CR 2 |  |
| 8 | TT > UE | Send REGISTRATION ACCEPT | The REGISTRATION ACCEPT message contains: - 5G-GUTI: "24408300010266434444" - 5GS TAI list with TAI with: - TAI: 244/083/000001 |  |  |
| 9 | UE > TT | Send REGISTRATION COMPLETE | The UE sends a REGISTRATION COMPLETE and stops the timer T3510 |  |  |
| 10 | TT | Bring down Cell A Bring up Cell B |  |  |  |
| 11 | UE > TT | Send REGISTRATION REQUEST | The UE sends a REGISTRATION REQUEST to the NG-SS with 5GS registration type IE as "mobility registration updating" or as "initial registration" and 5GS mobile identity information element type "5G-GUTI" and starts the timer T3510 | CR 2 |  |
| NOTE: a verification of timer states as indicated in steps 7), 9) and 11) is not required. An explicit verification may be possible on MEs providing the related information. |  |  |  |  |  |

#### 5.3.9.5	Acceptance criteria

CR 1 can be verified by a method explicitly verifying the correct execution of the READ commands in step 2) on the listed EFs (A.2/1 or A.2/2).

CR 2 is implicitly verified if the REGISTRATION REQUEST in step 7) contains the following parameters:

5G-GUTI:						244 083 00010266436587

Last visited registered TAI:	244/083/000002

and in step 11):

5G-GUTI:						244 083 00010266434444

Last visited registered TAI:	244/083/000001

### 5.3.10	UE identification after SUPI is changed

#### 5.3.10.1	Definition and applicability

A globally unique temporary user identity for 5GS-based services, the 5G globally unique temporary identity (5G-GUTI), is used for identification within the signalling procedures. A UE supporting N1 mode includes a valid 5G-GUTI, if any is available, in the REGISTRATION REQUEST and DEREGISTRATION REQUEST messages.

#### 5.3.10.2	Conformance requirement

CR 1	The following 5GMM parameters shall be stored on the USIM if the corresponding file is present:

-	5G-Globally Unique Temporary Identifier (5G-GUTI);

-	Last visited registered Tracking Area Identity in 5GS (TAI);

-	5GS update status;

-	5G NAS security context parameters from a full native 5G NAS security context.

NOTE:	The presence and format of corresponding files on the USIM is specified in 3GPP TS 31.102 [4].

Reference:

-	TS 31.102 [19], clauses 4.4.11.2

CR 2	If the corresponding file is not present on the USIM, these 5GMM parameters are stored in a non-volatile memory in the ME together with the SUPI from the USIM. These 5GMM parameters can only be used if the SUPI from the USIM matches the SUPI stored in the non-volatile memory; else the UE shall delete the 5GMM parameters.

Reference:

-	TS 24.501 [25], clauses 5.3.3, 5.5.1.2 and Annex C.

CR 3	The ME correctly performs the READ BINARY command on EFIMSI.

References:

-	TS 31 101 [33], clause 11.1.3;

-	ETSI TS 102 221 [8], clause 11.1.3 and 14.1.1.

#### 5.3.10.3	Test purpose

The purpose of this test is to verify that:

- the READ EFIMSI commands are performed correctly by the ME;

- the ME deletes the 5GMM parameters from non-volatile memory in case SUPI is changed.

#### 5.3.10.4	Method of test

##### 5.3.10.4.1	Initial conditions

The values of the E-UTRAN UICC defined in clause 4.5.3 of the present document are used with EFIMSI as defined in clause 4.6.5.

The TT (NG-SS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):		244/083/000001

-	Access control:				unrestricted.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and runs an initial activation.

##### 5.3.10.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | READ EFIMSI |  | CR 3 | A.2/1 OR A.2/2 |
| 2 | UE > TT | Send REGISTRATION REQUEST | The UE sends a REGISTRATION REQUEST to the NG-SS with 5GS registration type IE as "initial registration" |  |  |
| 3 | TT > UE | Send REGISTRATION ACCEPT | The REGISTRATION ACCEPT message contains: - 5G-GUTI: "24408300010266436587" - 5GS TAI list with TAI with: - TAI: 244/083/000001 |  |  |
| 4 | UE > TT | Send REGISTRATION COMPLETE |  |  |  |
| 5 | USER/TT | Power off/deactivate the UE |  |  |  |
| 6 | UE | Change UICC configuration | The IMSI is set to: "246081685533963" |  |  |
| 7 | USER/TT | Run activation of the UE |  |  |  |
| 8 | UE > TT | Send REGISTRATION REQUEST | The UE sends a REGISTRATION REQUEST to the NG-SS | CR 1 CR 2 |  |

#### 5.3.10.5	Acceptance criteria

CR 1 and CR 2 are implicitly verified in the REGISTRATION REQUESTs in step 8). The used E-UTRAN UICC shall not be capable to store any 5G related information. Therefore, none of the information provided in step 3) shall be available second REGISTRATION REQUEST.

### 5.3.11	SUCI calculation by ME using Profile A

#### 5.3.11.1	Definition and applicability

If the operator's decision is that the ME shall calculate the SUCI, the Home Network Operator shall provision a list of the Protection Scheme Identifiers that the operator allows in the USIM. The list of Protection Scheme Identifiers in the USIM may contain one or more Protection Scheme Identifiers in order of their priority. The ME shall read the SUCI calculation information from the USIM, including the SUPI, the Home Network Public Key, the Home Network Public Key Identifier, and the list of Protection Scheme Identifiers. The ME shall select the protection scheme from its supported schemes that has the highest priority in the list obtained from the USIM.

#### 5.3.11.2	Conformance requirement

CR 1	SUCI calculation procedure shall be performed by the ME if Service n°124 is available and Service n°125 is not available in EFUST

Reference:

-	TS 31.102 [19], clause 5.3.47

CR 2	As part of the SUCI calculation performed by the ME, the ME performs the reading procedure with EFSUCI_Calc_Info.

Reference:

-	TS 31.102 [19], clauses 4.4.11.8 and 5.3.47.

CR 3	The ME shall calculate the SUCI using the ECIES scheme profile A if highest priority of the protection schemes listed in the USIM is the ECIES scheme profile A.

References:

-	TS 31 101 [33], clause 11.1.3;

-	ETSI TS 102 221 [8], clause 11.1.3 and 14.1.1.

#### 5.3.11.3	Test purpose

The purpose of this test is to verify that:

- the READ EFRouting_Indicator, EFSUCI_Calc_Info and EFIMSI commands are performed correctly by the ME;

- To verify that the ME performs SUCI calculation procedure using the profile with the highest priority (i.e. ECIES scheme profile A and the Home Network Public Key).

#### 5.3.11.4	Method of test

##### 5.3.11.4.1	Initial conditions

The values of the default 5G-NR UICC defined in clause 4.5.9 of the present document are used with EFIMSI as defined in clause 4.6.5 and the following exceptions:

EFSUCI_Calc_Info (Subscription Concealed Identifier Calculation Information EF)

Logically:

Protection Scheme Identifier List data object:

Protection Scheme Identifier 1 – ECIES scheme profile A

Key Index 1:		1

Protection Scheme Identifier 2 – ECIES scheme profile B

Key Index 2:		2

Protection Scheme Identifier 3 – null-scheme

Key Index 3:		0

Home Network Public Key List data object

Home Network Public Key 1 Identifier:	30

Home Network Public Key 1:

-	5A 8D 38 86 48 20 19 7C 33 94 B9 26 13 B2 0B 91 63 3C BD 89 71 19 27 3B F8 E4 A6 F4 EE C0 A6 50

Home Network Public Key 2 Identifier:	27

Home Network Public Key 2:

-	04 72 DA 71 97 62 34 CE 83 3A 69 07 42 58 67 B8 2E 07 4D 44 EF 90 7D FB 4B 3E 21 C1 C2 25 6E BC D1 5A 7D ED 52 FC BB 09 7A 4E D2 50 E0 36 C7 B9 C8 C7 00 4C 4E ED C4 F0 68 CD 7B F8 D3 F9 00 E3 B4

The NG-SS shall be configured with Home Network Private Key as following (for Profile A):

| Coding: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | C5 | 3C | 22 | 20 | 8B | 61 | 86 | 0B |
|  | B9 | B10 | B11 | B12 | B13 | B14 | B15 | B16 |
|  | 06 | C6 | 2E | 54 | 06 | A7 | B3 | 30 |
|  | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | C2 | B5 | 77 | AA | 55 | 58 | 98 | 15 |
|  | B25 | B26 | B27 | B28 | B29 | B30 | B31 | B32 |
|  | 10 | D1 | 28 | 24 | 7D | 38 | BD | 1D |

The TT (NG-SS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):		244/083/000001

-	Access control:				unrestricted.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and runs an initial activation.

##### 5.3.11.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | READ EFIMSI, EFUST, EFRouting_Indicator and EFSUCI_Calc_Info | (Evaluation of service settings) | CR 2 | A.2/1 OR A.2/2 |
| 2 | ME | Perform SUCI calculation | The ME performs a SUCI calculation using Profile A | (CR 3) |  |
| 3 | UE > TT | Send REGISTRATION REQUEST | The UE sends a REGISTRATION REQUEST to the NG-SS with 5GS registration type IE as "initial registration" and 5GS mobile identity information element type "SUCI" | CR 1 |  |
| 4 | TT > UE | Send REGISTRATION ACCEPT |  |  |  |
| 5 | UE > TT | Send REGISTRATION COMPLETE |  |  |  |

#### 5.3.11.5	Acceptance criteria

CR 1 and CR 3 are implicitly verified in the REGISTRATION REQUEST in step 3). The requirements are met if the REGISTRATION REQUEST sent to the NG-SS contains:

SUPI format:						0

Home Network Identifier:		246/081

Routing indicator:				17

Protection scheme id:			01

Home network public key Id:	30

Scheme output:					ECC ephemeral public key, encryption of 357935793 and MAC tag value

CR 2 can be verified by a method explicitly verifying the correct execution of the READ commands in step 1) on the listed EFs (A.2/1 or A.2/2).

### 5.3.12	UE identification by SUCI during initial registration – SUCI calculation by USIM using profile A

#### 5.3.12.1	Definition and applicability

If the operator's decision, indicated by the USIM, is that the USIM shall calculate the SUCI, then the USIM shall not give the ME any parameter for the calculation of the SUCI including the Home Network Public Key Identifier, the Home Network Public Key, and the Protection Scheme Identifier. If the ME determines that the calculation of the SUCI, indicated by the USIM, shall be performed by the USIM, the ME shall delete any previously received or locally cached parameters for the calculation of the SUCI including the Routing Indicator, the Home Network Public Key Identifier, the Home Network Public Key and the Protection Scheme Identifier.

#### 5.3.12.2	Conformance requirement

CR 1	SUCI calculation procedure shall be performed by the USIM if Services n°124 and n°125 are available in EFUST.

Reference:

-	TS 31.102 [19], clause 5.3.48;

-	TS 33.501 [24], clause Annex C.

CR 2	The ME shall use the GET IDENTITY command in SUCI context to retrieve the SUCI calculated by the USIM.

Reference:

-	TS 31.102 [19], clause 5.3.48 and 7.5;

-	TS 24.501 [42], clause 5.5.1.2.2.

CR 3	This GET IDENTITY command shall be as per 7.5.2 in TS 31.102

Reference:

-	TS 31.102 [4], clause 7.5.

#### 5.3.12.3	Test purpose

The purpose of this test is to verify that:

1)	the GET IDENTITY command is performed correctly by the ME.

2)	the UE includes the SUCI received from the 5G-NR UICC within GET IDENTITY response in the 5GS mobile identity IE.

#### 5.3.12.4	Method of test

##### 5.3.12.4.1	Initial conditions

The values of the default 5G-NR UICC defined in clause 4.5.9 of the present document are used with EFIMSI as defined in clause 4.6.5 with the following exceptions:

EFUST (USIM Service Table)

Logically:

| Service n°125: |  | SUCI calculation by the USIM | available |
| --- | --- | --- | --- |

Coding:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary: | xxxx xx1x | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxxx xxxx | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | … | B16 |  |  |  |
|  | xxxx xxxx | xxxx xxxx | xx11 xxxx | ... | xxx1 111x |  |  |  |

5G-NR UICC is configured with:

Protection Scheme Identifier:				ECIES scheme profile A

Key Index:									1

Home Network Public Key Identifier:	30

Home Network Public Key:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 5A | 8D | 38 | 86 | 48 | 20 | 19 | 7C |
|  | B9 | B10 | B11 | B12 | B13 | B14 | B15 | B16 |
|  | 33 | 94 | B9 | 26 | 13 | B2 | 0B | 91 |
|  | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 63 | 3C | BD | 89 | 71 | 19 | 27 | 3B |
|  | B25 | B26 | B27 | B28 | B29 | B30 | B31 | B32 |
|  | F8 | E4 | A6 | F4 | EE | C0 | A6 | 50 |

EFSUCI_Calc_Info (Subscription Concealed Identifier Calculation Information EF):	Not available to the ME

The NG-SS shall be configured with Home Network Private Key for profile A:

| Coding: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | C5 | 3C | 22 | 20 | 8B | 61 | 86 | 0B |
|  | B9 | B10 | B11 | B12 | B13 | B14 | B15 | B16 |
|  | 06 | C6 | 2E | 54 | 06 | A7 | B3 | 30 |
|  | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | C2 | B5 | 77 | AA | 55 | 58 | 98 | 15 |
|  | B25 | B26 | B27 | B28 | B29 | B30 | B31 | B32 |
|  | 10 | D1 | 28 | 24 | 7D | 38 | BD | 1D |

The TT (NG-SS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):		244/083/000001

-	Access control:				unrestricted.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and runs an initial activation.

##### 5.3.12.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | READ EFUST | (Evaluation of service settings) |  |  |
| 2 | USIM | Perform SUCI calculation | The USIM performs a SUCI calculation using Profile A | (CR 1) |  |
| 3 | ME > USIM | Send GET IDENTITY | The GET IDENTITY command is sent with Identity Context in P2 as SUCI (0x01). The USIM returns the calculated SUCI | (CR 1) CR 2 CR3 | A.2/1 OR A.2/2 |
| 4 | UE > TT | Send REGISTRATION REQUEST | The UE sends a REGISTRATION REQUEST with 5GS registration type IE as "initial registration" and 5GS mobile identity information element type "SUCI" | CR 1 (CR 2) (CR 3) |  |
| 5 | TT > UE | Send REGISTRATION ACCEPT | The TT sends a REGISTRATION ACCEPT with a 5G-GUTI |  |  |
| 6 | UE > TT | Send REGISTRATION COMPLETE |  |  |  |

#### 5.3.12.5	Acceptance criteria

CR 1 is implicitly verified if the REGISTRATION REQUEST in step 4). The requirements are met if the REGISTRATION REQUEST sent to the NG-SS contains:

SUPI format:						0

Home Network Identifier:		246/081

Routing indicator:				17

Protection scheme id:			01

Home network public key Id:	30

Scheme output:					ECC ephemeral public key, encryption of 357935793 and MAC tag value

CR 2 and CR  3 are explicitly verified in step 3). The requirements are met if the GET IDENTITY command in step 3) is performed correctly.

An implicit verification of CR 2 and CR  3 can be done in step 4). The requirements are met if the REGISTRATION REQUEST sent to the NG-SS contains what is defined to fulfil CR 1.

### 5.3.13	SUCI calculation by ME using null scheme– no Protection Scheme Identifier provisioned in the USIM

#### 5.3.13.1	Definition and applicability

If the operator's decision is that ME shall calculate the SUCI, and the home network operator has not provisioned any Protection Scheme Identifier definition in the list of Protection Scheme Identifiers in the USIM, the ME shall calculate the SUCI using the null-scheme.

#### 5.3.13.2	Conformance requirement

CR 1	SUCI calculation procedure shall be performed by the ME if Service n°124 is available and Service n°125 is not available in EFUST

Reference:

-	TS 31.102 [19], clause 5.3.47

CR 2	As part of the SUCI calculation performed by the ME, the ME performs the reading procedure with EFRouting_Indicator

Reference:

-	TS 31.102 [19], clauses 4.4.11.11 and 5.3.51.

CR 3	The ME shall calculate the SUCI using the null-scheme if no Protection Scheme Identifier is provisioned in the USIM.

References:

-	TS 33.501 [24], clause 6.12.2 and Annex C

#### 5.3.13.3	Test purpose

The purpose of this test is to verify that:

1)	the READ EFSUCI_Calc_Info, EFRouting_Indicator and EFIMSI commands are performed correctly by the ME.

2)	the ME performs the SUCI calculation procedure using null-scheme.

#### 5.3.13.4	Method of test

##### 5.3.13.4.1	Initial conditions

The values of the default 5G-NR UICC defined in clause 4.5.9 of the present document are used with EFIMSI as defined in clause 4.6.5 with the following exceptions:

EFSUCI_Calc_Info (Subscription Concealed Identifier Calculation Information EF)

Logically:

null

Coding:

| Byte: | B1 | B2 |
| --- | --- | --- |
| Hex | A0 | 00 |

The TT (NG-SS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):		244/083/000001

-	Access control:				unrestricted.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and runs an initial activation.

##### 5.3.13.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | READ EFUST, EFRouting_Indicator and EFSUCI_Calc_Info | (Evaluation of data and service settings) | CR 1 (CR 2) | A.2/1 OR A.2/2 |
| 2 | ME | Perform SUCI calculation | The ME performs a SUCI calculation using the null scheme | (CR 2) |  |
| 3 | UE > TT | Send REGISTRATION REQUEST | The UE sends a REGISTRATION REQUEST with 5GS registration type IE as "initial registration" and 5GS mobile identity information element type "SUCI" | (CR 1) CR 2 CR 3 |  |
| 4 | TT > UE | Send REGISTRATION ACCEPT | The TT sends a REGISTRATION ACCEPT with a 5G-GUTI |  |  |
| 5 | UE > TT | Send REGISTRATION COMPLETE |  |  |  |

#### 5.3.13.5	Acceptance criteria

CR 1 can be verified by a method explicitly verifying the correct execution of the READ commands in step 1) on the listed EFs (A.2/1 or A.2/2). CR 2 may optionally be verified by the used explicit method too.

CR 2 and CR 3 (and CR 1) are implicitly verified in the REGISTRATION REQUEST in step 3). The requirements are met if the REGISTRATION REQUEST sent to the NG-SS contains:

SUPI format:						0

Home Network Identifier:		246/081

Routing indicator:				17

Protection scheme id:			00

Home network public key Id:	0

Scheme output:					357935793

### 5.3.14	SUCI calculation by ME using null scheme – no Home Network Public Key for supported protection scheme provisioned in the USIM

#### 5.3.14.1	Definition and applicability

If the operator's decision is that ME shall calculate the SUCI, and the home network operator has not provisioned the Home Network Public Key for the protection scheme configured in the USIM that the ME supports, the ME shall calculate the SUCI using the null-scheme.

#### 5.3.14.2	Conformance requirement

CR 1	SUCI calculation procedure shall be performed by the ME if Service n°124 is available and Service n°125 is not available in EFUST.

Reference:

-	TS 31.102 [19], clause 5.3.47

CR 2	As part of the SUCI calculation performed by the ME, the ME performs the reading procedure with EFSUCI_Calc_Info and EFRouting_Indicator

Reference:

-	TS 31.102 [19], clauses 4.4.11.8, 4.4.11.11, 5.3.47 and 5.3.51.

CR 3	The ME shall calculate the SUCI using the null scheme if no Home Network Public Key configured in the USIM for the protection scheme configured in the USIM that the ME supports.

References:

-	TS 33.501 [24], clause 6.12.2 and Annex C

-	TS 24.501 [25], clause 5.5.1.2.4.

#### 5.3.14.3	Test purpose

The purpose of this test is to verify that:

1)	the READ EFSUCI_Calc_Info, EFRouting_Indicator and EFIMSI commands are performed correctly by the ME.

2)	the ME performs the SUCI calculation procedure using null-scheme.

#### 5.3.14.4	Method of test

##### 5.3.14.4.1	Initial conditions

The values of the default 5G-NR UICC defined in clause 4.5.9 of the present document are used with EFIMSI as defined in clause 4.6.5 with the following exceptions:

EFSUCI_Calc_Info (Subscription Concealed Identifier Calculation Information EF)

Logically:

Protection Scheme Identifier List data object

Protection Scheme Identifier	1 – ECIES scheme profile B

Key Index 1:						0

Coding:

| Byte: | B1 | B2 | B3 | B4 |
| --- | --- | --- | --- | --- |
| Hex | A0 | 02 | 02 | 00 |

The TT (NG-SS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):		244/083/000001

-	Access control:				unrestricted.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and runs an initial activation.

##### 5.3.14.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | READ EFUST, EFRouting_Indicator and EFSUCI_Calc_Info | (Evaluation of data and service settings) | CR 1 (CR 2) | A.2/1 OR A.2/2 |
| 2 | ME | Perform SUCI calculation | The ME performs a SUCI calculation using the null scheme | (CR 2) |  |
| 3 | UE > TT | Send REGISTRATION REQUEST | The UE sends a REGISTRATION REQUEST with 5GS registration type IE as "initial registration" and 5GS mobile identity information element type "SUCI" | (CR 1) CR 2 CR 3 |  |
| 4 | TT > UE | Send REGISTRATION ACCEPT | The TT sends a REGISTRATION ACCEPT with a 5G-GUTI |  |  |
| 5 | UE > TT | Send REGISTRATION COMPLETE |  |  |  |

#### 5.3.14.5	Acceptance criteria

CR 1 can be verified by a method explicitly verifying the correct execution of the READ commands in step 1) on the listed EFs (A.2/1 or A.2/2). CR 2 may optionally be verified by the used explicit method too.

CR 2 and CR 3 (and CR 1) are implicitly verified in the REGISTRATION REQUEST in step 3). The requirements are met if the REGISTRATION REQUEST sent to the NG-SS contains:

SUPI format:						0

Home Network Identifier:		246/081

Routing indicator:				17

Protection scheme id:			00

Home network public key Id:	0

Scheme output:					357935793

### 5.3.15	SUCI calculation by ME using null scheme with the E-UTRAN/EPC UICC

#### 5.3.15.1	Definition and applicability

If the operator's decision is that ME shall calculate the SUCI, the home network operator shall provision the Protection Scheme and public key in the USIM that the operator allows. But if the SUCI calculation indication is not present, the calculation is in the ME. If the Home Network Public Key or the priority list are not provisioned in the USIM, the ME shall calculate the SUCI using the null-scheme. The Routing Indicator shall be stored in the USIM. If the Routing Indicator is not present in the USIM, the ME shall set it to a default value 0.

#### 5.3.15.2	Conformance requirement

CR 1	SUCI calculation procedure shall be performed by the ME if the SUCI calculation indication is not present in the USIM.

Reference:

-	TS 33.501 [24], clause 5.2.5, 6.12.2, Annex C.

CR 2	The ME shall calculate the SUCI using the null-scheme if E-UTRAN/EPC UICC is installed into the ME.

-	TS 33.501 [24], clause 6.12.2 and Annex C;

-	TS 24.501 [25], clause 5.5.1.2.4 and 9.11.3.4.

#### 5.3.15.3	Test purpose

The purpose of this test is to verify that the ME performs the SUCI calculation procedure using null-scheme.

#### 5.3.15.4	Method of test

##### 5.3.15.4.1	Initial conditions

The values of the E-UTRAN UICC defined in clause 4.5.4 of the present document are used with the following exceptions:

EFIMSI (International Mobile Subscriber Identity)

Logically:

246081357935793

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 08 | 29 | 64 | 80 | 31 | 75 | 39 | 75 | 39 |

If activated, the TT (NG-SS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):		244/083/000001

-	Access control:				unrestricted.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and runs an initial activation. The TT (NG-SS) activates Cell A.

##### 5.3.15.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | READ EFUST | (Evaluation of service settings) |  |  |
| 2 | UE > TT | Send REGISTRATION REQUEST | The UE sends a REGISTRATION REQUEST to the NG-SS |  |  |
| 3 | TT > UE | Send REGISTRATION REJECT | The TT sends a REGISTRATION REJECT with cause: Roaming not allowed in this tracking area |  |  |
| 4 | UE | Deactivate and then re-activate the UE |  |  |  |
| 5 | UE | READ EFUST, EFIMSI | (Evaluation of data and service settings) |  |  |
| 6 | ME | Perform SUCI calculation | The ME performs a SUCI calculation using the null scheme | (CR 1) (CR 2) |  |
| 7 | UE > TT | Send REGISTRATION REQUEST | The UE sends a REGISTRATION REQUEST with 5GS registration type IE as "initial registration" and 5GS mobile identity information element type "SUCI" | CR 1 CR 2 |  |
| 8 | TT > UE | Send REGISTRATION ACCEPT | The TT sends a REGISTRATION ACCEPT with a 5G-GUTI |  |  |
| 9 | UE > TT | Send REGISTRATION COMPLETE |  |  |  |

#### 5.3.15.5	Acceptance criteria

CR 1 and CR 2 are implicitly verified in the REGISTRATION REQUEST in step 7). The requirements are met if the REGISTRATION REQUEST sent to the NG-SS contains:

SUPI format:						0

Home Network Identifier:		246/081

Routing indicator:				0

Protection scheme id:			00

Home network public key Id:	0

Scheme output:					357935793

### 5.3.16	SUCI calculation by ME using the lower priority protection scheme when the higher priority protection scheme is not supported by the ME

#### 5.3.16.1	Definition and applicability

If the operator's decision is that the ME shall calculate the SUCI, the Home Network Operator shall provision a list of the Protection Scheme Identifiers that the operator allows in the USIM. The list of Protection Scheme Identifiers in the USIM may contain one or more Protection Scheme Identifiers in order of their priority. The ME shall read the SUCI calculation information from the USIM, including the SUPI, the Home Network Public Key, the Home Network Public Key Identifier, and the list of Protection Scheme Identifiers. The ME shall select the protection scheme from its supported schemes that has the highest priority in the list obtained from the USIM. If the higher priority protection scheme is not supported by the ME, the ME should use the lower priority protection scheme to calculate the SUCI.

#### 5.3.16.2	Conformance requirement

CR 1	SUCI calculation procedure shall be performed by the ME if Service n°124 is available and Service n°125 is not available in EFUST.

Reference:

-	TS 31.102 [19], clause 5.3.47 and 5.3.51.

CR 2	As part of the SUCI calculation performed by the ME, the ME performs the reading procedure with EFSUCI_Calc_Info.

Reference:

-	TS 31.102 [19], clauses 4.4.11.8 and 5.3.47.

CR 3	The ME shall select the protection scheme from its supported schemes that has the highest priority in the list are obtained from the USIM. If the higher priority protection scheme is not supported by the ME, the ME should use the lower priority protection scheme to calculate the SUCI.

References:

-	TS 31.102 [19], clauses 4.4.11.8;

-	TS 33.501 [24], clause 6.12.2 and Annex C

-	TS 24.501 [25], clause 5.5.1.2.4.

#### 5.3.16.3	Test purpose

The purpose of this test is to verify that:

- the READ EFRouting_Indicator, EFSUCI_Calc_Info and EFIMSI commands are performed correctly by the ME.

- if the higher priority protection scheme is not supported by the ME, the ME should use the lower priority protection scheme to calculate the SUCI.

#### 5.3.16.4	Method of test

##### 5.3.16.4.1	Initial conditions

The values of the default 5G-NR UICC defined in clause 4.5.9 of the present document are used with EFIMSI as defined in clause 4.6.5 with the following exceptions:

EFSUCI_Calc_Info (Subscription Concealed Identifier Calculation Information EF)

Logically:

Protection Scheme Identifier List data object

Protection Scheme Identifier	1 – any value of the reserved range (i.e 0x3 - 0xB) that is not standardized

Key Index 1:				1

Protection Scheme Identifier	2 – ECIES scheme profile A

Key Index 1:				2

Protection Scheme Identifier	3 – null-scheme

Key Index 3:						0

Home Network Public Key List data object

Home Network Public Key 1 Identifier:	16

Home Network Public Key 1:

- 2E 85 DA EC 6A C9 B5 2B 5D 2D 58 02 33 29 57 75 49 44 5A 39 3D 2A 68 E6 12 14 27 34 95 AD BE 65

Home Network Public Key 2 Identifier:	30

Home Network Public Key 2:

- 5A 8D 38 86 48 20 19 7C 33 94 B9 26 13 B2 0B 91 63 3C BD 89 71 19 27 3B F8 E4 A6 F4 EE C0 A6 50

Coding:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | A0 | 06 | NOTE | 01 | 01 | 02 | 00 | 00 |
|  | B9 | B10 | B11 | B12 | B13 | B14 | B15 | B16 |
|  | A1 | 4A | 80 | 01 | 10 | 81 | 20 | 2E |
|  | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 85 | DA | EC | 6A | C9 | B5 | 2B | 5D |
|  | B25 | B26 | B27 | B28 | B29 | B30 | B31 | B32 |
|  | 2D | 58 | 02 | 33 | 29 | 57 | 75 | 49 |
|  | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 |
|  | 44 | 5A | 39 | 3D | 2A | 68 | E6 | 12 |
|  | B41 | B42 | B43 | B44 | B45 | B46 | B47 | B48 |
|  | 14 | 27 | 34 | 95 | AD | BE | 65 | 80 |
|  | B49 | B50 | B51 | B52 | B53 | B54 | B55 | B56 |
|  | 01 | 1E | 81 | 20 | 5A | 8D | 38 | 86 |
|  | B57 | B58 | B59 | B60 | B61 | B62 | B63 | B64 |
|  | 48 | 20 | 19 | 7C | 33 | 94 | B9 | 26 |
|  | B65 | B66 | B67 | B68 | B69 | B70 | B71 | B72 |
|  | 13 | B2 | 0B | 91 | 63 | 3C | BD | 89 |
|  | B73 | B74 | B75 | B76 | B77 | B78 | B79 | B80 |
|  | 71 | 19 | 27 | 3B | F8 | E4 | A6 | F4 |
|  | B81 | B82 | B83 | B84 |
|  | EE | C0 | A6 | 50 |

NOTE:	Any value of the reserved range (i.e 0x3 - 0xB) that is not standardized (e.g. 0xB).

The NG-SS shall be configured with Home Network Private Key as following (for Profile A):

| Coding: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | C5 | 3C | 22 | 20 | 8B | 61 | 86 | 0B |
|  | B9 | B10 | B11 | B12 | B13 | B14 | B15 | B16 |
|  | 06 | C6 | 2E | 54 | 06 | A7 | B3 | 30 |
|  | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | C2 | B5 | 77 | AA | 55 | 58 | 98 | 15 |
|  | B25 | B26 | B27 | B28 | B29 | B30 | B31 | B32 |
|  | 10 | D1 | 28 | 24 | 7D | 38 | BD | 1D |

The TT (NG-SS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):		244/083/000001

-	Access control:				unrestricted.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and runs an initial activation.

##### 5.3.16.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | READ EFIMSI, EFUST, EFRouting_Indicator and EFSUCI_Calc_Info | (Evaluation of data and service settings) | CR 2 | A.2/1 OR A.2/2 |
| 2 | ME | Perform SUCI calculation | The ME performs a SUCI calculation | (CR 1) (CR 3) |  |
| 3 | UE > TT | Send REGISTRATION REQUEST | The UE sends a REGISTRATION REQUEST with 5GS registration type IE as "initial registration" and 5GS mobile identity information element type "SUCI" | CR 1 CR 3 |  |
| 4 | TT > UE | Send REGISTRATION ACCEPT | The TT sends a REGISTRATION ACCEPT with a 5G-GUTI |  |  |
| 5 | UE > TT | Send REGISTRATION COMPLETE |  |  |  |

#### 5.3.16.5	Acceptance criteria

CR 1 can be verified by a method explicitly verifying the correct execution of the READ commands in step 1) on the listed EFs (A.2/1 or A.2/2).

CR 1 and CR 3 are implicitly verified in the REGISTRATION REQUEST in step 3). The requirements are met if the REGISTRATION REQUEST sent to the NG-SS contains:

SUPI format:						0

Home Network Identifier:		246/081

Routing indicator:				17

Protection scheme id:			01

Home network public key Id:	30

Scheme output:					ECC ephemeral public key, encryption of 357935793 and MAC tag value

### 5.3.17	SUCI calculation by ME using Profile B with compressed Home Network Public Key

#### 5.3.17.1	Definition and applicability

If the operator's decision is that ME shall calculate the SUCI, the home network operator shall provision a list of the Protection Scheme Identifiers that the operator allows in the USIM. The list of Protection Scheme Identifiers in the USIM may contain one or more Protection Scheme Identifiers in order of their priority. The ME shall read the SUCI calculation information from the USIM, including the SUPI, the Home Network Public Key, the Home Network Public Key Identifier, and the list of Protection Scheme Identifiers. The ME shall select the protection scheme from its supported schemes that has the highest priority in the list obtained from the USIM.

#### 5.3.17.2	Conformance requirement

CR 1	SUCI calculation procedure shall be performed by the ME if Service n°124 is available and Service n°125 is not available in EFUST.

Reference:

-	TS 31.102 [19], clause 5.3.47 and 5.3.51.

CR 2	As part of the SUCI calculation performed by the ME, the ME performs the reading procedure with EFSUCI_Calc_Info.

Reference:

-	TS 31.102 [19], clauses 4.4.11.8 and 5.3.47.

CR 3	The ME shall calculate the SUCI using the highest priority supported protection scheme and the home network public key stored on the USIM.

References:

-	TS 31.102 [19], clauses 4.4.11.8;

-	TS 33.501 [24], clause 6.12.2 and Annex C

-	TS 24.501 [25], clause 5.5.1.2.4.

CR 4	The ME shall be capable to calculate the SUCI using Profile B with the ECC public key provided in compressed format.

References:

-	TS 33.501 [24], Annex C.3.4.2 and Annex C.4.4

-	RFC 5480 [51], clause 2.2.

#### 5.3.17.3	Test purpose

The purpose of this test is to verify that:

- the READ EFRouting_Indicator, EFSUCI_Calc_Info and EFIMSI commands are performed correctly by the ME.

- the ME performs the SUCI calculation procedure using the profile with the highest priority (i.e. ECIES scheme profile B and the home network public key)

#### 5.3.17.4	Method of test

##### 5.3.17.4.1	Initial conditions

The values of the default 5G-NR UICC defined in clause 4.5.9 of the present document are used with EFIMSI as defined in clause 4.6.5 with the following exceptions:

EFSUCI_Calc_Info (Subscription Concealed Identifier Calculation Information EF)

Logically:

Protection Scheme Identifier List data object

Protection Scheme Identifier	1 – ECIES scheme profile B

Key Index 1:				1

Protection Scheme Identifier	2 – ECIES scheme profile A

Key Index 1:				2

Protection Scheme Identifier	3 – null-scheme

Key Index 3:						0

Home Network Public Key List data object

Home Network Public Key 1 Identifier:	27

Home Network Public Key 1 (see note):

-	02 72 DA 71 97 62 34 CE 83 3A 69 07 42 58 67 B8 2E 07 4D 44 EF 90 7D FB 4B 3E 21 C1 C2 25 6E BC D1

Home Network Public Key 2 Identifier:	30

Home Network Public Key 2:

- 5A 8D 38 86 48 20 19 7C 33 94 B9 26 13 B2 0B 91 63 3C BD 89 71 19 27 3B F8 E4 A6 F4 EE C0 A6 50

- Coding:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | A0 | 06 | 02 | 01 | 01 | 02 | 00 | 00 |
|  | B9 | B10 | B11 | B12 | B13 | B14 | B15 | B16 |
|  | A1 | 4B | 80 | 01 | 1B | 81 | 21 | 02 |
|  | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 72 | DA | 71 | 97 | 62 | 34 | CE | 83 |
|  | B25 | B26 | B27 | B28 | B29 | B30 | B31 | B32 |
|  | 3A | 69 | 07 | 42 | 58 | 67 | B8 | 2E |
|  | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 |
|  | 07 | 4D | 44 | EF | 90 | 7D | FB | 4B |
|  | B41 | B42 | B43 | B44 | B45 | B46 | B47 | B48 |
|  | 3E | 21 | C1 | C2 | 25 | 6E | BC | D1 |
|  | B49 | B50 | B51 | B52 | B53 | B54 | B55 | B56 |
|  | 80 | 01 | 1E | 81 | 20 | 5A | 8D | 38 |
|  | B57 | B58 | B59 | B60 | B61 | B62 | B63 | B64 |
|  | 86 | 48 | 20 | 19 | 7C | 33 | 94 | B9 |
|  | B65 | B66 | B67 | B68 | B69 | B70 | B71 | B72 |
|  | 26 | 13 | B2 | 0B | 91 | 63 | 3C | BD |
|  | B73 | B74 | B75 | B76 | B77 | B78 | B79 | B80 |
|  | 89 | 71 | 19 | 27 | 3B | F8 | E4 | A6 |
|  | B81 | B82 | B83 | B84 | B85 |
|  | F4 | EE | C0 | A6 | 50 |

- NOTE:	EFSUCI_Calc_Info contains the compressed form of the ECC public key for Profile B.

The NG-SS shall be configured with Home Network Private Key as following:

| Coding: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | F1 | AB | 10 | 74 | 47 | 7E | BC | C7 |
|  | B9 | B10 | B11 | B12 | B13 | B14 | B15 | B16 |
|  | F5 | 54 | EA | 1C | 5F | C3 | 68 | B1 |
|  | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 61 | 67 | 30 | 15 | 5E | 00 | 41 | AC |
|  | B25 | B26 | B27 | B28 | B29 | B30 | B31 | B32 |
|  | 44 | 7D | 63 | 01 | 97 | 5F | EC | DA |

The TT (NG-SS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):		244/083/000001

-	Access control:				unrestricted.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and runs an initial activation.

##### 5.3.17.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | READ EFIMSI, EFUST, EFRouting_Indicator and EFSUCI_Calc_Info | (Evaluation of data and service settings) | CR 2 | A.2/1 OR A.2/2 |
| 2 | ME | Perform SUCI calculation | The ME performs a SUCI calculation | (CR 1) (CR 3) (CR 4) |  |
| 3 | UE > TT | Send REGISTRATION REQUEST | The UE sends a REGISTRATION REQUEST with 5GS registration type IE as "initial registration" and 5GS mobile identity information element type "SUCI" | CR 1 CR 3 CR 4 |  |
| 4 | TT > UE | Send REGISTRATION ACCEPT | The TT sends a REGISTRATION ACCEPT with a 5G-GUTI |  |  |
| 5 | UE > TT | Send REGISTRATION COMPLETE |  |  |  |

#### 5.3.17.5	Acceptance criteria

CR 1 can be verified by a method explicitly verifying the correct execution of the READ commands in step 1) on the listed EFs (A.2/1 or A.2/2).

CR 1 and CR 3 are implicitly verified in the REGISTRATION REQUEST in step 3). The requirements are met if the REGISTRATION REQUEST sent to the NG-SS contains:

SUPI format:						0

Home Network Identifier:		246/081

Routing indicator:				17

Protection scheme id:			02

Home network public key Id:	27

Scheme output:					ECC ephemeral public key, encryption of 357935793 and MAC tag value

## 5.4	Unified Access Control information handling for 5G-NR

### 5.4.1	Unified Access Control – Access identity 0, no access identities indicated by USIM

#### 5.4.1.1	Definition and applicability

The purpose of Unified Access Control procedure is to perform access barring check for a 5GS access attempt associated with a given Access Category and one or more Access Identities upon request from upper layers or the RRC layer.

The 5G network shall be able to broadcast barring control information (i.e. a list of barring parameters associated with an Access Identity and an Access Category) in SIB1.

If no Access Identities are configured in EFUAC_AIC and in EFACC, Access Identity 0 is applicable. The UE shall read EFUAC_AIC and EFACC as part of USIM Initialization procedure.

The UE shall be able to determine whether or not a particular new access attempt is allowed based on barring parameters that the UE receives from the broadcast barring control information and the configuration in the USIM.

When the NAS detects an access event, the NAS shall perform the mapping of the kind of request to one or more access identities and one access category and lower layers will perform access barring checks for that request based on the determined access identities and access category.

#### 5.4.1.2	Conformance requirement

CR 1	The ME shall read the access control value as part of the USIM-ME initialization procedure, and subsequently adopt this value.

Reference:

-	TS 31.102 [19], clause 5.1.1.2.

CR 2	Access Identities are configured at the UE as listed in TS 22.261 [36] Table 6.22.2.2-1. Access Categories are defined by the combination of conditions related to UE and the type of access attempt as listed in TS 22.261 [36] Table 6.22.2.3-1. One or more Access Identities and only one Access Category are selected and tested for an access attempt.

Reference:

-	TS 22.261 [36], clause 6.22.2.

CR 3	The UE shall be able to determine whether or not a particular new access attempt is allowed based on uac-BarringInfo broadcast in SIB1. Access Control check shall be performed as per the information received in uac-BarringInfoSetList.

Reference:

-	TS 38.331 [37], clause 5.3.14.

#### 5.4.1.3	Test purpose

The purpose of this test is to verify that:

1)	the ME reads the access control value from EFUAC-AIC and EFACC as part of the USIM-ME initialisation procedure, and subsequently adopts this value.

2)	the UE maps the kind of request to one or more access identities and one access category and lower layers performs access barring checks for that request based on the determined access identities and access category.

#### 5.4.1.4	Method of test

##### 5.4.1.4.1	Initial conditions

The values of the 5G-NR UICC as defined in clause 4.5.9 of the present document are used with EFIMSI as defined in clause 4.6.5 and the following exceptions:

EFUAC_AIC and EFACC

No Access Identities configured in EFUAC_AIC and no Access Classes configured in EFACC as shown in table 5.4.1-1.

EFUST (USIM Service Table)

Logically:

Settings from clause 4.5.9 (5G-NR UICC) of the present document apply with the following changes:

| Service n°124 |  | Subscription identifier privacy support | not available |
| --- | --- | --- | --- |
| Service n°126 |  | UAC Access Identities support | available |

Coding:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary: | xxxx xx1x | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxx1 xx1x | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | … | B16 |  |  |  |
|  | xxxx xxxx | xxxx xxxx | xx11 xxxx | ... | xx10 011x |  |  |  |

The NG-SS is configured to transmit on the BCCH with the following parameters:

Cell A:

-	TAI (MCC/MNC/TAC):		MCC, MNC: see table 5.4.1-1, TAC: 000001.

-	CellIdentity:					000000001

For the table 5.4.1-1:

uac-BarringInfo in SIB1 should be set as in the table:

-	Refer to Annex A for the Methods UAC_BarringInfo_xxxxxx() in the table.

If present in the REGISTRATION ACCEPT, the 5GS network feature support IE indicates Access identities 1 and 2 are not valid.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case.

##### 5.4.1.4.2	Test sequence settings

Table 5.4.1-1

| TC Seq# | Access Category | USIM |  | SIB1 on Cell A |  | REGISTRATION ACCEPT (5GS network feature support IE) |  | Result |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | EFUAC_AIC | EFACC (Byte 1 b8-b4) | uac-BarringInfo | PLMN-Identity (MCC/MNC) | MPS indicator Bit | MCS indicator Bit | Registration successful? | MO Data call successful? |
| 1.1 | 7 | 0x00 00 00 00 | 00000 | Not Present | 246 / 081 | 0 | 0 | Yes | Yes |
| 1.2 | 3 | 0x00 00 00 00 | 00000 | UAC_BarringInfo_Common (3,0x0000000'B) | 246 / 081 | 0 | 0 | No | N/A |
| 1.3 | 3 | 0x00 00 00 00 | 00000 | UAC_BarringInfo_PerPLMN (3,0x0000000'B) | 246 / 081 | 0 | 0 | No | N/A |
| 1.4 | 3 | 0x00 00 00 00 | 00000 | UAC_BarringInfo_PerPLMN (3,0x1000000'B) | 246 / 081 | 0 | 0 | No | N/A |
| 1.5 | 7 | 0x00 00 00 00 | 00000 | UAC_BarringInfo_Common (7,0x0000000'B) | 246 / 081 | 0 | 0 | Yes | No |
| 1.6 | 7 | 0x00 00 00 00 | 00000 | UAC_BarringInfo_PerPLMN (7,0x0000000'B) | 246 / 081 | 0 | 0 | Yes | No |
| 1.7 | 3 | 0x00 00 00 00 | 00000 | UAC_BarringInfo_Common2 (7,0x1000000'B, 3,0x0000000'B) | 244 / 081 | 0 | 0 | No | N/A |

##### 5.4.1.4.3	Procedure

Procedure/steps to be repeated for all sequences listed in table 5.4.1-1:

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | TT | SIB1 of Cell A is transmitted as defined in the initial conditions for tests from table 5.4.1-1 | Barring info is set as in table 5.4.1-1 See Annex A for the Methods UAC_BarringInfo_xxxxxx() in the table |  |  |
| 2 | UE | Power UE on | The UE performs a SIM initialisation. (includes reading of EFUAC-AIC and EFACC) | CR 1 | A.2/1 OR A.2/2 |

| If cell access is not allowed: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 3 | UE > TT | REGISTRATION REQUEST in not sent | See column ‘Registration successful?’ for the result | CR 2 CR 3 |  |
| 4 | User | End test sequence |  |  |  |

| If cell access is allowed: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 3 | UE > TT | Send REGISTRATION REQUEST | See column ‘Registration successful?’ for the result | CR 2 CR 3 |  |
| 4 | TT > UE | Send REGISTRATION ACCEPT | For simplicity other signalling is not shown |  |  |

| If MO Data call is not allowed: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 5 | UE | Attempt to set up MO Data call | To set up the MO Data call the MMI or EMMI is used | CR 2 CR 3 |  |
| 6 | User | End test sequence |  |  |  |

| If MO Data call is allowed: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 5 | UE > TT | Set up MO Data call (PDU SessionEstablishment) | To set up the MO Data call the MMI or EMMI is used | CR 2 CR 3 |  |
| 6 | User | End test sequence |  |  |  |

#### 5.4.1.5	Acceptance criteria

For the scenarios in table 5.4.1-1:

CR 1 is explicitly verified at step 2) by analysing the file READ commands for EFUAC-AIC and EFACC during the USIM-ME initialisation procedure.

CR 2 and CR 3 are verified at:

- steps 3) and 5) by analysing if the UE shall make a successful or unsuccessful Registration to the network in accordance with the result indicated in the table and

- steps 4) and 5) by analysing if the UE shall make a successful or unsuccessful MO Data call in accordance with the result indicated in the table if the step is applicable.

### 5.4.1A	Unified Access Control – Access identity 0, no access identities indicated by USIM, Access Category 8

#### 5.4.1A.1	Definition and applicability

The purpose of Unified Access Control procedure is to perform access barring check for a 5GS access attempt associated with a given Access Category and one or more Access Identities upon request from upper layers or the RRC layer.

The 5G network shall be able to broadcast barring control information (i.e. a list of barring parameters associated with an Access Identity and an Access Category) in SIB1.

If no Access Identities are configured in EFUAC_AIC and in EFACC, Access Identity 0 is applicable. The UE shall read EFUAC_AIC and EFACC as part of USIM Initialization procedure.

The UE shall be able to determine whether or not a particular new access attempt is allowed based on barring parameters that the UE receives from the broadcast barring control information and the configuration in the USIM.

When the NAS detects an access event, the NAS shall perform the mapping of the kind of request to one or more access identities and one access category and lower layers will perform access barring checks for that request based on the determined access identities and access category.

If RRC state is RRC_INACTIVE and the resumption of the RRC connection is triggered due to an RNA Update, RRC layer shall select Access Category as '8' and perform unified access control procedure in case there is no ongoing emergency service.

#### 5.4.1A.2	Conformance requirement

CR 1	The ME shall read the access control value as part of the USIM-ME initialization procedure, and subsequently adopt this value.

Reference:

-	TS 31.102 [19], clause 5.1.1.2.

CR 2	Access Identities are configured at the UE as listed in TS 22.261 [36] Table 6.22.2.2-1. Access Categories are defined by the combination of conditions related to UE and the type of access attempt as listed in TS 22.261 [36] Table 6.22.2.3-1. One or more Access Identities and only one Access Category are selected and tested for an access attempt.

Reference:

-	TS 22.261 [36], clause 6.22.2.

CR 3	The UE shall be able to determine whether or not a particular new access attempt is allowed based on uac-BarringInfo broadcast in SIB1. Access Control check shall be performed as per the information received in uac-BarringInfoSetList.

Reference:

-	TS 38.331 [37], clause 5.3.14.

CR 4	If the resumption of the RRC connection is triggered due to an RNA update and there is no ongoing emergency service RRC shall select '8' as the Access Category and perform the unified access control procedure.

Reference:38.331 [37], clause 5.3.13.2.

#### 5.4.1A.3	Test purpose

The purpose of this test is to verify that:

1)	the ME reads the access control value from EFUAC-AIC and EFACC as part of the USIM-ME initialisation procedure, and subsequently adopts this value.

2)	the UE maps the kind of request to one or more access identities and one access category and lower layers performs access barring checks for that request based on the determined access identities and access category.

3)	the UE performs unified access control procedure if RNA Update procedure is triggered.

#### 5.4.1A.4	Method of test

##### 5.4.1A.4.1	Initial conditions

The values of the 5G-NR UICC as defined in clause 4.5.9 of the present document are used with EFIMSI as defined in clause 4.6.5 and the following exceptions:

EFUAC_AIC and EFACC

No Access Identities configured in EFUAC_AIC and no Access Classes configured in EFACC as shown in table 5.4.1A-1.

EFUST (USIM Service Table)

Logically:

Settings from clause 4.5.9 (5G-NR UICC) of the present document apply with the following changes:

| Service n°124 |  | Subscription identifier privacy support | not available |
| --- | --- | --- | --- |
| Service n°126 |  | UAC Access Identities support | available |

Coding:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary: | xxxx xx1x | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxx1 xx1x | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | … | B16 |  |  |  |
|  | xxxx xxxx | xxxx xxxx | xx11 xxxx | ... | xx10 011x |  |  |  |

The TT (NG-SS) is configured to transmit on the BCCH with the following parameters:

Cell A:

-	TAI (MCC/MNC/TAC):		MCC, MNC: see table 5.4.1A-1, TAC: 000001.

-	CellIdentity:					000000001

For the table 5.4.1A-1:

No uac-BarringInfo in SIB1.

If present in the REGISTRATION ACCEPT, the 5GS network feature support IE indicates Access identities 1 and 2 are not valid.

Cell B:

-	TAI (MCC/MNC/TAC):		MCC, MNC: see table 5.4.1A-1, TAC 000001.

-	CellIdentity:					000000002

For the table 5.4.1A-1:

uac-BarringInfo in SIB1 should be set as in the table:

-	Refer to Annex A for the Methods UAC_BarringInfo_xxxxxx() in the table.

If present in the REGISTRATION ACCEPT, the 5GS network feature support IE indicates Access identities 1 and 2 are not valid.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case.

##### 5.4.1A.4.2	Test sequence settings

Table 5.4.1A-1

| TC Seq# | Access Category | USIM |  | SIB1 on Cell A |  | REGISTRATION ACCEPT (5GS network feature support IE) |  | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | EFUAC_AIC | EFACC (Byte 1 b8-b4) | uac-BarringInfo | PLMN-Identity (MCC/MNC) | MPS indicator Bit | MCS indicator Bit | RRCResumeRequest with resumeCause set to rna-Update successful? |
| 2.1 | 8 | 0x00 00 00 00 | 00000 | Not Present | 246 / 081 | 0 | 0 | Yes |
| 2.2 | 8 | 0x00 00 00 00 | 00000 | UAC_BarringInfo_Common (8,0x0000000'B) | 246 / 081 | 0 | 0 | No |
| 2.3 | 8 | 0x00 00 00 00 | 00000 | UAC_BarringInfo_PerPLMN (8,0x0000000'B) | 246 / 081 | 0 | 0 | No |
| 2.4 | 8 | 0x00 00 00 00 | 00000 | UAC_BarringInfo_Common2 (7,0x1000000'B, 8,0x0000000'B) | 244 / 081 | 0 | 0 | No |

##### 5.4.1A.4.3	Procedure

Procedure/steps to be repeated for all sequences listed in table 5.4.1A-1:

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | TT | SIB1 of Cell A is transmitted as defined in the initial conditions for tests from table 5.4.1A-1 | No barring info is provided to Cell A |  |  |
| 2 | UE | Power UE on | The UE performs a SIM initialisation (includes reading of EFUAC-AIC and EFACC) | CR 1 | A.2/1 OR A.2/2 |
| 3 | UE > TT | Send REGISTRATION REQUEST |  |  |  |
| 4 | TT > UE | Send REGISTRATION ACCEPT | The registration is successful |  |  |
| 5 | UE > TT | Set up MO Data call (PDU SessionEstablishment) | To set up the MO Data call the MMI or EMMI is used. The MO Data call is set up successfully |  |  |
| 6 | TT > UE | Send RRC RELEASE with suspendConfig in criticalExtensions (with the choice rrcRelease) | ran-NotificationAreaInfo in suspendConfig contains the cellList with cellIdentity of Cell A: - cellList { - plmn-Identity {mcc, mnc}, - -- see table 5.4.1A-1 for MCC/MNC - ran-AreaCells 000000001’B - } |  |  |
| 7 | TT | Deactivate Cell A and activate Cell B |  |  |  |
| 8 | TT | SIB1 of Cell B is transmitted as defined in table 5.4.1A-1 | Barring info is as in the table. See Annex A for the Methods UAC_BarringInfo_xxxxxx() in the table |  |  |
| 9 | UE > TT | Initiate RRC RESUME REQUEST procedure with resumeCause set to rna-Update. | See the column ‘RRC RESUME REQUEST with resumeCause set to rna-Update successful?’ for the result | CR 2 CR 3 CR 4 |  |

#### 5.4.1A.5	Acceptance criteria

For the scenarios in table 5.4.1A-1:

CR 1 is explicitly verified at step 2) by analysing the file READ commands for EFUAC-AIC and EFACC during the USIM-Terminal initialisation procedure.

CR 2, CR 3, and CR 4 are verified at steps 9) and 10) by analysing if the UE shall make a successful or unsuccessful RRC Resumption for RNA Update in accordance with the result indicated in the table.

### 5.4.2	Unified Access Control – Access Identity 1 – MPS indicated by USIM

#### 5.4.2.1	Definition and applicability

The purpose of Unified Access Control procedure is to perform access barring check for a 5GS access attempt associated with a given Access Category and one or more Access Identities upon request from upper layers or the RRC layer.

The 5G network shall be able to broadcast barring control information (i.e. a list of barring parameters associated with an Access Identity and an Access Category) in SIB1.

The EFUAC-AIC in the USIM contains the configuration information pertaining to access identity 1 allocated for high priority services MPS.

The UE shall be able to determine whether or not a particular new access attempt is allowed based on barring parameters that the UE receives from the broadcast barring control information and the configuration in the USIM.

When the NAS detects an access event, the NAS shall perform the mapping of the kind of request to one or more access identities and one access category and lower layers will perform access barring checks for that request based on the determined access identities and access category.

#### 5.4.2.2	Conformance requirement

CR 1	The ME shall read the access control value as part of the USIM-ME initialization procedure, and subsequently adopt this value.

Reference:

-	TS 31.102 [19], clause 5.1.1.2.

CR 2	Access Identities are configured at the UE as listed in TS 22.261 [36] Table 6.22.2.2-1. Access Categories are defined by the combination of conditions related to UE and the type of access attempt as listed in TS 22.261 [36] Table 6.22.2.3-1. One or more Access Identities and only one Access Category are selected and tested for an access attempt.

Reference:

-	TS 22.261 [36], clause 6.22.2

CR 3	Access Identity 1 is valid if the RPLMN is the HPLMN, EHPLMN or visited PLMN of the home country.

Reference:

-	TS 24.501 [25], clause 4.5.2

CR 4	The UE shall be able to determine whether or not a particular new access attempt is allowed based on uac-BarringInfo broadcast in SIB1. Access Control check shall be performed as per the information received in uac-BarringInfoSetList.

Reference:

-	TS 38.331 [37], clauses 5.3.14

#### 5.4.2.3	Test purpose

The purpose of this test is to verify that:

1)	the ME reads the access control value from EFUAC-AIC and EFACC as part of the USIM-ME initialisation procedure, and subsequently adopts this value.

2)	the UE maps the kind of request to one or more access identities and one access category and lower layers performs access barring checks for that request based on the determined access identities and access category.

3)	the UE determines whether or not a particular access attempt is allowed based on uac-BarringInfo broadcast in SIB1 and if the RPLMN is the HPLMN, EHPLMN or visited PLMN of the home country.

#### 5.4.2.4	Method of test

##### 5.4.2.4.1	Initial conditions

The values of the 5G-NR UICC as defined in clause 4.5.9 of the present document are used with EFIMSI as defined in clause 4.6.5 and the following exceptions:

EFUAC_AIC and EFACC

No Access Identities configured in EFUAC_AIC and no Access Classes configured in EFACC as also shown in table 5.4.2-1.

EFUST (USIM Service Table)

Logically:

Settings from clause 4.5.9 (5G-NR UICC) of the present document apply with the following changes:

| Service n°124 |  | Subscription identifier privacy support | not available |
| --- | --- | --- | --- |
| Service n°126 |  | UAC Access Identities support | available |

Coding:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary: | xxxx xx1x | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxx1 xx1x | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | … | B16 |  |  |  |
|  | xxxx xxxx | xxxx xxxx | xx11 xxxx | ... | xx10 011x |  |  |  |

The TT (NG-SS) is configured to transmit on the BCCH with the following parameters:

Cell A:

-	TAI (MCC/MNC/TAC):		MCC, MNC: see table 5.4.2-1, TAC: 000001.

-	CellIdentity:					000000001

For the table 5.4.2-1:

uac-BarringInfo in SIB1 should be set as in the table:

-	Refer to Annex A for the Methods UAC_BarringInfo_xxxxxx() in the table.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case.

##### 5.4.2.4.2	Test sequence settings

Table 5.4.2-1

| TC Seq# | Access Category | USIM |  | SIB1 on Cell A |  | REGISTRATION ACCEPT (5GS network feature support IE) |  | Result |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | EFUAC_AIC | EFACC (Byte 1 b8-b4) | uac-BarringInfo | PLMN-Identity (MCC/MNC) | MPS indicator Bit | MCS indicator Bit | Registration successful? | MO Data call successful? |
| 1.1 | 7 | 0x01 00 00 00 | 00000 | Not Present | 246 / 081 | 0 | 0 | Yes | Yes |
| 1.2 | 3 | 0x01 00 00 00 | 00000 | UAC_BarringInfo_Common (3,0x1000000'B) | 246 / 081 | 0 | 0 | No | NA |
| 1.3 | 3 | 0x01 00 00 00 | 00000 | UAC_BarringInfo_PerPLMN (3,0x1000000'B) | 246 / 081 | 0 | 0 | No | NA |
| 1.4 | 3 | 0x01 00 00 00 | 00000 | UAC_BarringInfo_Common (3,0x1000000'B) | 246 / 082 | 0 | 0 | No | NA |
| 1.5 | 3 | 0x01 00 00 00 | 00000 | UAC_BarringInfo_Common (3,0x0100000'B) | 246 / 081 | 0 | 0 | Yes | Yes |
| 1.6 | 3 | 0x03 00 00 00 | 00000 | UAC_BarringInfo_Common (3,0x0100000'B) | 246 / 081 | 0 | 0 | Yes | Yes |
| 1.7 | 7 | 0x01 00 00 00 | 00000 | UAC_BarringInfo_Common (7,0x0000000'B) | 246 / 081 | 0 | 0 | Yes | Yes |
| 1.8 | 3 | 0x01 00 00 00 | 00000 | UAC_BarringInfo_Common (3,0x0000000'B) | 244 / 081 | 0 | 0 | No | NA |
| 1.9 | 7 | 0x01 00 00 00 | 00000 | UAC_BarringInfo_Common (7,0x1000000'B) | 246 / 081 | 0 | 0 | Yes | No |
| 1.10 | 7 | 0x01 00 00 00 | 00000 | UAC_BarringInfo_PerPLMN (7,0x1000000'B) | 246 / 081 | 0 | 0 | Yes | No |
| 1.11 | 7 | 0x01 00 00 00 | 00000 | UAC_BarringInfo_Common (7,0x1000000'B) | 246 / 082 | 0 | 0 | Yes | No |
| 1.12 | 7 | 0x01 00 00 00 | 00000 | UAC_BarringInfo_Common (7,0x0000000'B) | 244 / 081 | 0 | 0 | Yes | No |
| 1.13 | 7 | 0x01 00 00 00 | 00000 | UAC_BarringInfo_Common (7,0x01000000'B) | 244 / 081 | 1 | 0 | Yes | Yes |
| 1.14 | 7 | 0x02 00 00 00 | 00000 | UAC_BarringInfo_Common (7,0x01000000'B) | 244 / 081 | 1 | 0 | Yes | Yes |
| 1.15 | 7 | 0x01 00 00 00 | 00000 | UAC_BarringInfo_Common2 (3,0x0100000'B, 7,0x1000000'B) | 246 / 081 | 0 | 0 | Yes | No |
| 1.16 | 3 | 0x01 00 00 00 | 00000 | UAC_BarringInfo_Common2 (7,0x1000000'B, 3,0x0000000'B) | 244 / 081 | 0 | 0 | No | NA |
| 2.5 | 8 | 0x01 00 00 00 | 00000 | UAC_BarringInfo_Common2 (7,0x1000000'B, 8,0x0000000'B) | 244 / 081 | 0 | 0 | No |  |
| 2.6 | 8 | 0x01 00 00 00 | 00000 | UAC_BarringInfo_Common (8,0x0000000'B) | 246 / 081 | 0 | 0 | Yes |  |

##### 5.4.2.4.3	Procedure

Procedure/steps to be repeated for all sequences listed in table 5.4.2-1:

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | TT | SIB1 of Cell A is transmitted as defined in the initial conditions for tests from table 5.4.2-1 | Barring info is set as in table 5.4.2-1 See Annex A for the Methods UAC_BarringInfo_xxxxxx() in the table |  |  |
| 2 | UE | Power UE on | The UE performs a SIM initialisation. (includes reading of EFUAC-AIC and EFACC) | CR 1 | A.2/1 OR A.2/2 |

| If cell access is not allowed: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 3 | UE > TT | REGISTRATION REQUEST is not sent | See column ‘Registration successful?’ for the result | CR 2 CR 3 CR 4 |  |
| 4 | User | End test sequence |  |  |  |

| If cell access is allowed: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 3 | UE > TT | Send REGISTRATION REQUEST | See column ‘Registration successful?’ for the result | CR 2 CR 3 CR 4 |  |
| 4 | TT > UE | Send REGISTRATION ACCEPT | For simplicity other signalling is not shown |  |  |

| If MO Data call is not allowed: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 5 | UE | Attempt to set up MO Data call | To set up the MO Data call the MMI or EMMI is used | CR 2 CR 3 CR 4 |  |
| 6 | User | End test sequence |  |  |  |

| If MO Data call is allowed: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 5 | UE > TT | Set up MO Data call (PDU SessionEstablishment) | To set up the MO Data call the MMI or EMMI is used | CR 2 CR 3 CR 4 |  |
| 6 | User | End test sequence |  |  |  |

#### 5.4.2.5	Acceptance criteria

For the scenarios in table 5.4.2-1:

CR 1 is explicitly verified at step 2) by analysing the file READ commands for EFUAC-AIC and EFACC during the USIM-ME initialisation procedure.

CR 2, CR 3, and CR 4 are verified at:

- step 3) by analysing if the UE shall make a successful or unsuccessful registration to the network in accordance with the result indicated in the table and

- step 5) by analysing if the UE shall make a successful or unsuccessful MO Data call in accordance with the result indicated in the table if the step is applicable.

### 5.4.2A	Unified Access Control – Access Identity 1 – MPS indicated by USIM, Access Category 8

#### 5.4.2A.1	Definition and applicability

The purpose of Unified Access Control procedure is to perform access barring check for a 5GS access attempt associated with a given Access Category and one or more Access Identities upon request from upper layers or the RRC layer.

The 5G network shall be able to broadcast barring control information (i.e. a list of barring parameters associated with an Access Identity and an Access Category) in SIB1.

The EFUAC-AIC in the USIM contains the configuration information pertaining to access identity 1 allocated for high priority services MPS.

The UE shall be able to determine whether or not a particular new access attempt is allowed based on barring parameters that the UE receives from the broadcast barring control information and the configuration in the USIM.

When the NAS detects an access event, the NAS shall perform the mapping of the kind of request to one or more access identities and one access category and lower layers will perform access barring checks for that request based on the determined access identities and access category.

If RRC state is RRC_INACTIVE and the resumption of the RRC connection is triggered due to an RNA Update RRC layer shall select Access Category as '8' and perform unified access control procedure in case there is no ongoing emergency service.

#### 5.4.2A.2	Conformance requirement

CR 1	The ME shall read the access control value as part of the USIM-ME initialization procedure, and subsequently adopt this value.

Reference:

-	TS 31.102 [19], clause 5.1.1.2.

CR 2	Access Identities are configured at the UE as listed in TS 22.261 [36] Table 6.22.2.2-1. Access Categories are defined by the combination of conditions related to UE and the type of access attempt as listed in TS 22.261 [36] Table 6.22.2.3-1. One or more Access Identities and only one Access Category are selected and tested for an access attempt.

Reference:

-	TS 22.261 [36], clause 6.22.2

CR 3	Access Identity 1 is valid if the RPLMN is the HPLMN, EHPLMN or visited PLMN of the home country.

Reference:

-	TS 24.501 [25], clause 4.5.2

CR 4	The UE shall be able to determine whether or not a particular new access attempt is allowed based on uac-BarringInfo broadcast in SIB1. Access Control check shall be performed as per the information received in uac-BarringInfoSetList.

Reference:

-	TS 38.331 [37], clauses 5.3.14

CR 5	If the resumption of the RRC connection is triggered due to an RNA update and there is no ongoing emergency service RRC shall select '8' as the Access Category and perform the unified access control procedure.

Reference:

-	TS 38.331 [37], clauses 5.3.13.2

#### 5.4.2A.3	Test purpose

The purpose of this test is to verify that:

1)	the ME reads the access control value from EFUAC-AIC and EFACC as part of the USIM-ME initialisation procedure, and subsequently adopts this value.

2)	the UE maps the kind of request to one or more access identities and one access category and lower layers performs access barring checks for that request based on the determined access identities and access category.

3)	the UE determines whether or not a particular access attempt is allowed based on uac-BarringInfo broadcast in SIB1 and if the RPLMN is the HPLMN, EHPLMN or visited PLMN of the home country.

4)	the UE performs unified access control procedure if RNA Update procedure is triggered.

#### 5.4.2A.4	Method of test

##### 5.4.2A.4.1	Initial conditions

The values of the 5G-NR UICC as defined in clause 4.5.9 of the present document are used with EFIMSI as defined in clause 4.6.5 and the following exceptions:

EFUAC_AIC and EFACC

No Access Identities configured in EFUAC_AIC and no Access Classes configured in EFACC as also shown in table 5.4.2A-2.

EFUST (USIM Service Table)

Logically:

| Service n°124 |  | Subscription identifier privacy support | not available |
| --- | --- | --- | --- |
| Service n°126 |  | UAC Access Identities support | available |

Coding:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary: | xxxx xx1x | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxx1 xx1x | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | … | B16 |  |  |  |
|  | xxxx xxxx | xxxx xxxx | xx11 xxxx | ... | xx10 011x |  |  |  |

The TT (NG-SS) is configured to transmit on the BCCH with the following parameters:

Cell A:

-	TAI (MCC/MNC/TAC):		MCC, MNC: see table 5.4.2A-1, TAC: 000001.

-	CellIdentity:					000000001

For the table 5.4.2A-1:

No uac-BarringInfo in SIB1.

Cell B:

-	TAI (MCC/MNC/TAC):		MCC, MNC: see table 5.4.2A-1, TAC: 000001.

-	CellIdentity:					000000002

For the table 5.4.2A-1:

uac-BarringInfo in SIB1 should be set as in the table:

-	Refer to Annex A for the Methods UAC_BarringInfo_xxxxxx() in the table.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case.

##### 5.4.2A.4.2	Test sequence settings

Table 5.4.2A-1

| TC Seq# | Access Category | USIM |  | SIB1 on Cell B |  | REGISTRATION ACCEPT (5GS network feature support IE) |  | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | EFUAC_AIC | EFACC (Byte 1 b8-b4) | uac-BarringInfo | PLMN-Identity (MCC/MNC) | MPS indicator Bit | MCS indicator Bit | RRCResumeRequest with resumeCause set to rna-Update successful? |
| 2.1 | 8 | 0x01 00 00 00 | 00000 | Not Present | 246 / 081 | 0 | 0 | Yes |
| 2.2 | 8 | 0x01 00 00 00 | 00000 | UAC_BarringInfo_Common (8,0x1000000'B) | 246 / 081 | 0 | 0 | No |
| 2.3 | 8 | 0x01 00 00 00 | 00000 | UAC_BarringInfo_PerPLMN (8,0x0000000'B) | 246 / 081 | 0 | 0 | Yes |
| 2.4 | 8 | 0x01 00 00 00 | 00000 | UAC_BarringInfo_Common (8,0x1000000'B) | 244 / 081 | 1 | 0 | No |
| 2.5 | 8 | 0x01 00 00 00 | 00000 | UAC_BarringInfo_Common2 (7,0x1000000'B, 8,0x0000000'B) | 244 / 081 | 0 | 0 | No |
| 2.6 | 8 | 0x01 00 00 00 | 00000 | UAC_BarringInfo_Common (8,0x0000000'B) | 246 / 081 | 0 | 0 | Yes |

##### 5.4.2A.4.3	Procedure

Procedure/steps to be repeated for all sequences listed in table 5.4.2A-1:

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | TT | SIB1 of Cell A is transmitted as defined in the initial conditions and in table 5.4.2A-1 | No barring info is provided to Cell A |  |  |
| 2 | UE | Power UE on | The UE performs a SIM initialisation | CR 1 | A.2/1 OR A.2/2 |
| 3 | UE > TT | Send REGISTRATION REQUEST |  |  |  |
| 4 | TT > UE | Send REGISTRATION ACCEPT | The registration is successful |  |  |
| 5 | UE > TT | Set up MO Data call (PDU SessionEstablishment) | To set up the MO Data call the MMI or EMMI is used. The MO Data call is set up successfully |  |  |
| 6 | TT > UE | Send RRC RELEASE with suspendConfig in criticalExtensions (with the choice rrcRelease) | ran-NotificationAreaInfo in suspendConfig contains the cellList with cellIdentity of Cell A: - cellList { - plmn-Identity {mcc, mnc}, - -- see table 5.4.2A-1 for MCC/MNC - ran-AreaCells 000000001’B } |  |  |
| 7 | TT | Deactivate Cell A and activate Cell B |  |  |  |
| 8 | TT | SIB1 of Cell B is transmitted as defined in table 5.4.2A-1 | Barring info is as in the table. See Annex A for the Methods UAC_BarringInfo_xxxxxx() in the table |  |  |
| 9 | UE > TT | Initiate RRC RESUME REQUEST procedure with resumeCause set to rna-Update. | See the column ‘RRC RESUME REQUEST with resumeCause set to rna-Update successful?’ for the result | CR 2 CR 3 CR 4 CR 5 |  |

#### 5.4.2A.5	Acceptance criteria

For the scenarios in table 5.4. 2A-1:

CR 1 is explicitly verified at step 2) by analysing the file READ commands for EFUAC-AIC and EFACC during the USIM-Terminal initialisation procedure.

CR 2, CR 3, CR 4, and CR 5 are verified at step 9) by analysing if the UE shall make a successful or unsuccessful RRC Resumption for RNA Update in accordance with the result indicated in the table.

### 5.4.3	Unified Access Control – Access Identity 1 – no MPS indication by USIM and SUPI not changed

#### 5.4.3.1	Definition and applicability

The purpose of Unified Access Control procedure is to perform access barring check for a 5GS access attempt associated with a given Access Category and one or more Access Identities upon request from upper layers or the RRC layer.

The 5G network shall be able to broadcast barring control information (i.e. a list of barring parameters associated with an Access Identity and an Access Category) in SIB1.

When the UE is in the country of its HPLMN or in an EHPLMN (if the EHPLMN list is present), and the USIM file EFUAC_AIC does not indicate the UE is configured for access identity 1, the UE uses the MPS indicator bit of the 5GS network feature support IE in the REGISTRATION ACCEPT message to determine if access identity 1 is valid.

The UE shall not consider access identity 1 to be valid when the UE is not in the country of its HPLMN prior to receiving the MPS indicator bit of the 5GS network feature support IE in the REGISTRATION ACCEPT message being set to "Access identity 1 valid in RPLMN or equivalent PLMN".

The UE shall be able to determine whether or not a particular new access attempt is allowed based on barring parameters that the UE receives from the broadcast barring control information and the configuration in the USIM.

When the NAS detects an access event, the NAS shall perform the mapping of the kind of request to one or more access identities and one access category and lower layers will perform access barring checks for that request based on the determined access identities and access category.

#### 5.4.3.2	Conformance requirement

CR 1	The USIM file EFUAC_AIC does not indicate the UE is configured for access identity 1, the UE uses the MPS indicator bit of the 5GS network feature support IE in the REGISTRATION ACCEPT message to determine if access identity 1 is valid. shall read the access control value as part of the USIM-ME initialization procedure, and subsequently adopt this value.

Reference:

-	TS 24.501 [25], clause 4.5.2.

CR 2	The UE shall not consider access identity 1 to be valid when the UE is not in the country of its HPLMN prior to receiving the MPS indicator bit of the 5GS network feature support IE in the REGISTRATION ACCEPT message being set to "Access identity 1 valid in RPLMN or equivalent PLMN".

Reference:

-	TS 24.501 [25], clause 4.5.2.

CR 3	The MPS indicator is stored together with a PLMN identity of the PLMN that provided it, and is valid in that RPLMN or equivalent PLMN. The MPS indicator can only be used if the SUPI from the USIM matches the SUPI stored in the non-volatile memory of the ME, else the UE shall delete the MPS indicator.

Reference:

-	TS 24.501 [25], Annex C.

CR 4	Access Identity 1 is valid if the RPLMN is the HPLMN, EHPLMN or visited PLMN of the home country.

Reference:

-	TS 24.501 [25], clause 4.5.2

CR 5	Upon receiving a REGISTRATION ACCEPT message with the MPS indicator bit set to "Access identity 1 valid", the UE shall act as a UE with access identity 1 configured for MPS in all NG-RAN of the registered PLMN and its equivalent PLMNs. The MPS indicator bit in the 5GS network feature support IE provided in the REGISTRATION ACCEPT message is valid until the UE receives a REGISTRATION ACCEPT message with the MPS indicator bit set to "Access identity 1 not valid" or until the UE selects a non-equivalent PLMN. Access identity 1 is only applicable while the UE is in N1 mode.

Reference:

-	TS 24.501 [25], clause 5.5.1.3.4

CR 6	The UE shall be able to determine whether or not a particular new access attempt is allowed based on uac-BarringInfo broadcast in SIB1. Access Control check shall be performed as per the information received in uac-BarringInfoSetList.

Reference:

-	TS38.331 [37], clauses 5.3.14

#### 5.4.3.3	Test purpose

The purpose of this test is to verify that:

1)	if the RPLMN is the HPLMN, EHPLMN or visited PLMN of the home country and the USIM file EFUAC_AIC does not indicate the UE is configured for access identity 1 and the MPS indicator bit of the 5GS network feature support IE in the REGISTRATION ACCEPT message is received, UE considers access identity as 1 for access barring;

2)	the MPS indicator is stored together with a PLMN identity of the PLMN that provided it, and the MPS indicator can only be used if the SUPI from the USIM matches the SUPI stored in the non-volatile memory of the ME;

3)	the UE maps the kind of request to one or more access identities and one access category and lower layers performs access barring checks for that request based on the determined access identities and access category;

4)	the UE determines whether or not a particular access attempt is allowed based on uac-BarringInfo broadcast in SIB1 and if the RPLMN is the HPLMN, EHPLMN or visited PLMN of the home country;

5)	a UE operated with a USIM where the file EFUAC_AIC does not indicate access identity 1, after a change of the REGISTRATION ACCEPT message with the MPS indicator bit set to "Access identity 1 not valid", or when the UE has selected a non-equivalent PLMN, the UE is acting as if it is configured for access identity 0.

#### 5.4.3.4	Method of test

##### 5.4.3.4.1	Initial conditions

The values of the 5G-NR UICC as defined in clause 4.5.9 of the present document are used with EFIMSI as defined in clause 4.6.5 and the following exceptions:

EFUAC_AIC and EFACC

No Access Identity is configured in EFUAC_AIC and no Access Classes are configured in EFACC as given in table 5.4.3-1.

EFUST (USIM Service Table)

Logically:

| Service n°124 |  | Subscription identifier privacy support | not available |
| --- | --- | --- | --- |
| Service n°126 |  | UAC Access Identities support | available |

Coding:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary: | xxxx xx1x | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxxx xxxx | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | … | B16 |  |  |  |
|  | xxxx xxxx | xxxx xxxx | xx11 xxxx | ... | xx10 011x |  |  |  |

The TT (NG-SS) is configured to transmit on the BCCH with the following parameters:

Cell A:

-	TAI (MCC/MNC/TAC):		246/081/000001.

-	CellIdentity:					000000001

For Table 5.4.3-1:

After turning on the cell for the 2nd time, uac-BarringInfo in SIB1 should be set as in the table:

-	Refer to Annex A for the Methods UAC_BarringInfo_xxxxxx() in the table.

Cell B:

- TAI (MCC/MNC/TAC):	MCC, MNC: see table 5.4.3-1, TAC: 000002.

- CellIdentity:				000000002

For Table 5.4.3-1:

uac-BarringInfo in SIB1 should be set as in the table:

-	Refer to Annex A for the Methods UAC_BarringInfo_xxxxxx() in the table.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case.

##### 5.4.3.4.2	Test sequence settings

Table 5.4.3-1

| TC Seq# | Access Category | USIM |  | REGISTRATION ACCEPT (5GS network feature support IE) on Cell A at 1st power up of the ME |  | SIB1 of Cell A or B after the 2nd power up of the ME |  |  | Result |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | EFUAC_AIC | EFACC (Byte 1 b8-b4) | MPS indicator Bit | MCS indicator Bit | Cell A or B | uac-BarringInfo | PLMN-Identity (MCC/MNC) | Registration successful? | MO Data call successful? |
| 1.1 | 3 | 0x00 00 00 00 | 00000 | 1 | 0 | Cell A | Not Present | 246 / 081 | Yes | Yes |
| 1.2 | 3 | 0x00 00 00 00 | 00000 | 1 | 0 | Cell A | UAC_BarringInfo_Common (3,0x1000000'B) | 246 / 081 | No | N/A |
| 1.3 | 7 | 0x00 00 00 00 | 00000 | 1 | 0 | Cell A | UAC_BarringInfo_Common (7,0x0100000'B) | 246 / 081 | Yes | No |
| 1.4 | 3 | 0x00 00 00 00 | 00000 | 1 | 0 | Cell A | UAC_BarringInfo_Common (3,0x0100000'B) | 246 / 081 | Yes | Yes |
| 1.5 | 3 | 0x00 00 00 00 | 00000 | 1 | 0 | Cell B | UAC_BarringInfo_PerPLMN (3,0x0100000'B) | 246 / 082 | No | N/A |
| 1.6 | 3 | 0x00 00 00 00 | 00000 | 1 | 0 | Cell B | UAC_BarringInfo_Common (3,0x0000000'B) | 244 / 081 | No | N/A |
| 1.7 | 7 | 0x00 00 00 00 | 00000 | 1 | 0 | Cell B | UAC_BarringInfo_Common (7,0x0100000'B) | 244 / 081 | Yes | No |
| 1.8 | 7 | 0x00 00 00 00 | 00000 | 1 | 1 | Cell B | UAC_BarringInfo_Common (7,0x0100000'B) | 244 / 081 | Yes | No |

##### 5.4.3.4.3	Procedure

Procedure/steps to be repeated for all sequences listed in table 5.4.3-1:

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | TT | SIB1 of Cell A is transmitted without uac-BarringInfo in SIB1. |  |  |  |
| 2 | UE | Power UE on | The UE performs a SIM initialisation. (includes reading of EFUAC-AIC and EFACC) | CR 1 | A.2/1 OR A.2/2 |
| 3 | UE > TT | REGISTRATION REQUEST is sent |  |  |  |
| 4 | TT > UE | Send REGISTRATION ACCEPT | The MPS indicator bit set to "Access identity 1 valid". For simplicity other signalling is not shown |  |  |
| 5 | UE | Power UE off |  |  |  |
| 6 | TT | Deactivates Cell A |  |  |  |
| 7 | TT | Activates Cell A or B with SIB1 as specified in the table 5.4.3-1 | See the column 'SIB1 of Cell A or B after the 2nd power up of the ME' for the SIB 1 and the column 'Cell A or B' to find the cell to be activated for this test step. |  |  |
| 8 | UE | Power UE on | The UE performs a SIM initialisation again. |  |  |

| If cell access is not allowed: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 9 | UE > TT | REGISTRATION REQUEST in not sent | See column 'Registration successful?' for the result | CR 2 through CR 6 |  |
| 10 | USER | End test sequence |  |  |  |

| If cell access is allowed: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 9 | UE > TT | Send REGISTRATION REQUEST | See column ' Registration successful?' for the result | CR 2 through CR 6 |  |
| 10 | TT > UE | Send REGISTRATION ACCEPT | MPS and MCS indicator bits are set to zero in 5GS network feature support IE. For simplicity other signalling is not shown |  |  |

| If MO Data call is not allowed: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 11 | UE | Attempt to set up MO Data call | To set up the MO Data call the MMI or EMMI is used | CR 2 through CR 6 |  |
| 12 | USER | End test sequence |  |  |  |

| If MO Data call is allowed: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 11 | UE > TT | Set up MO Data call (PDU SessionEstablishment) | To set up the MO Data call the MMI or EMMI is used | CR 2 through CR 6 |  |
| 12 | USER | End test sequence |  |  |  |

#### 5.4.3.5	Acceptance criteria

For the scenarios in table 5.4.3-1:

CR 1 is explicitly verified at step 2) by analysing the file READ commands for EFUAC-AIC and EFACC during the USIM-Terminal initialisation procedure.

CR 2, CR 3, CR 4, CR 5, and CR 6 are verified

- at steps 9) and 10) by analysing if the UE shall make a successful or unsuccessful Registration to the network in accordance with the result indicated in the table and

- at steps 11) and 12) by analysing if the UE shall make a successful or unsuccessful MO Data call in accordance with the result indicated in the table if the step is applicable.

### 5.4.4	Unified Access Control – Access Identity 1 – no MPS indication by USIM and SUPI is changed

#### 5.4.4.1	Definition and applicability

The purpose of Unified Access Control procedure is to perform access barring check for a 5GS access attempt associated with a given Access Category and one or more Access Identities upon request from upper layers or the RRC layer.

The 5G network shall be able to broadcast barring control information (i.e. a list of barring parameters associated with an Access Identity and an Access Category) in SIB1.

When the UE is in the country of its HPLMN or in an EHPLMN (if the EHPLMN list is present), and the USIM file EFUAC_AIC does not indicate the UE is configured for access identity 1, the UE uses the MPS indicator bit of the 5GS network feature support IE in the REGISTRATION ACCEPT message to determine if access identity 1 is valid.

The MPS indicator is stored together with a PLMN identity of the PLMN that provided it and is valid in that RPLMN or equivalent PLMN. The MPS indicator can only be used if the SUPI from the USIM matches the SUPI stored in the non-volatile memory of the ME, else the UE shall delete the MPS indicator.

The UE shall be able to determine whether or not a particular new access attempt is allowed based on barring parameters that the UE receives from the broadcast barring control information and the configuration in the USIM.

When the NAS detects an access event, the NAS shall perform the mapping of the kind of request to one or more access identities and one access category and lower layers will perform access barring checks for that request based on the determined access identities and access category.

#### 5.4.4.2	Conformance requirement

CR 1	When the UE is in the country of its HPLMN, and the USIM file EFUAC_AIC does not indicate the UE is configured for access identity 1, the UE uses the MPS indicator bit of the 5GS network feature support IE in the REGISTRATION ACCEPT message to determine if access identity 1 is valid.

Reference:

-	TS 24.501 [25], clause 4.5.2.

CR 2	The UE shall not consider access identity 1 to be valid when the UE is not in the country of its HPLMN prior to receiving the MPS indicator bit of the 5GS network feature support IE in the REGISTRATION ACCEPT message being set to "Access identity 1 valid" in RPLMN or equivalent PLMN.

Reference:

-	TS 24.501 [25], clause 4.5.2.

CR 3	Upon receiving a REGISTRATION ACCEPT message with the MPS indicator bit set to "Access identity 1 valid" in RPLMN or equivalent PLMN, the UE shall act as a UE with access identity 1 configured for MPS in all NG-RAN of the registered PLMN and its equivalent PLMNs. The MPS indicator bit in the 5GS network feature support IE provided in the REGISTRATION ACCEPT message is valid until the UE receives a REGISTRATION ACCEPT message with the MPS indicator bit set to "Access identity 1 not valid in RPLMN or equivalent PLMN" or until the UE selects a non-equivalent PLMN. Access identity 1 is only applicable while the UE is in N1 mode.

Reference:

-	TS 24.501 [25], clause 5.5.1.2.4.

CR 4	The MPS indicator is stored together with a PLMN identity of the PLMN that provided it, and is valid in that RPLMN or equivalent PLMN. The MPS indicator can only be used if the SUPI from the USIM matches the SUPI stored in the non-volatile memory of the ME, else the UE shall delete the MPS indicator.

Reference:

-	TS 24.501 [25], Annex C.

CR 5	The UE shall be able to determine whether or not a particular new access attempt is allowed based on uac BarringInfo broadcast in SIB1. Access Control check shall be performed as per the information received in uac BarringInfoSetList.

Reference:

-	TS 38.331 [37], clause 5.3.14

#### 5.4.4.3	Test purpose

The purpose of this test is to verify that:

1)	if the RPLMN is the HPLMN, EHPLMN or visited PLMN of the home country and the USIM file EFUAC_AIC does not indicate the UE is configured for access identity 1 and the MPS indicator bit of the 5GS network feature support IE in the REGISTRATION ACCEPT message is received, UE considers access identity as 1 for access barring;

2)	the MPS indicator bit in the 5GS network feature support IE provided in the REGISTRATION ACCEPT message is valid until the UE receives a REGISTRATION ACCEPT message with the MPS indicator bit set to "Access identity 1 not valid" or until the UE selects a non-equivalent PLMN;

3)	the MPS indicator is stored together with a PLMN identity of the PLMN that provided it, and the MPS indicator can only be used if the SUPI from the USIM matches the SUPI stored in the non-volatile memory of the ME;

4)	the UE maps the kind of request to one or more access identities and one access category and lower layers performs access barring checks for that request based on the determined access identities and access category;

5)	a UE operated with a USIM where the file EFUAC_AIC does not indicate access identity 1, after a change of the REGISTRATION ACCEPT message with the MPS indicator bit set to "Access identity 1 not valid", or when the UE has selected a non-equivalent PLMN, the UE is acting as if it is configured for access identity 0.

#### 5.4.4.4	Method of test

##### 5.4.4.4.1	Initial conditions

The values of the 5G-NR UICC as defined in clause 4.5.9 of the present document are used with EFIMSI as defined in clause 4.6.5 and the following exceptions:

EFUAC_AIC and EFACC

No Access Identity is configured in EFUAC_AIC and no Access Classes are configured in EFACC as given in table 5.4.4-1.

EFUST (USIM Service Table)

Logically:

| Service n°124 |  | Subscription identifier privacy support | not available |
| --- | --- | --- | --- |
| Service n°126 |  | UAC Access Identities support | available |

Coding:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary: | xxxx xx1x | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxxx xxxx | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | … | B16 |  |  |  |
|  | xxxx xxxx | xxxx xxxx | xx11 xxxx | ... | xx10 011x |  |  |  |

The defined UICC/USIM data is installed on the UE.

The TT (NG-SS) is configured to transmit on the BCCH with the following parameters:

Cell A:

-	TAI (MCC/MNC/TAC):		246/081/000001.

-	CellIdentity:					000000001

For Table 5.4.4-1:

After turning on the cell for the 2nd time, uac-BarringInfo in SIB1 should be set as in the table:

-	Refer to Annex A for the Methods UAC_BarringInfo_xxxxxx() in the table.

Cell B:

Transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):		MCC, MNC:	see table 5.4.4-1, TAC: 000002.

-	CellIdentity:					000000002

For Table 5.4.4-1:

uac-BarringInfo in SIB1 should be set as in the table:

-	Refer to Annex A for the Methods UAC_BarringInfo_xxxxxx() in the table.

##### 5.4.4.4.2	Test sequence settings

Table 5.4.4-1

| TC Seq# | Access Category | USIM |  | REGISTRATION ACCEPT (5GS network feature support IE) on Cell A at 1st power up of the ME |  | SIB1 of Cell A or B after the 2nd power up of the ME |  |  | Result |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | EFUAC_AIC | EFACC (Byte 1 b8-b4) | MPS indicator Bit | MCS indicator Bit | Cell A or B | uac-BarringInfo | PLMN-Identity (MCC/MNC) | Registration successful? | MO Data call successful? |
| 1.1 | 3 | 0x00 00 00 00 | 00000 | 1 | 0 | Cell A | Not Present | 246 / 081 | Yes | Yes |
| 1.2 | 3 | 0x00 00 00 00 | 00000 | 1 | 0 | Cell A | UAC_BarringInfo_Common (3,0x0100000'B) | 246 / 081 | No | NA |
| 1.3 | 7 | 0x00 00 00 00 | 00000 | 1 | 0 | Cell A | UAC_BarringInfo_Common (7,0x0100000'B) | 246 / 081 | Yes | No |
| 1.4 | 3 | 0x00 00 00 00 | 00000 | 1 | 0 | Cell A | UAC_BarringInfo_Common (3,0x0000000'B) | 246 / 081 | No | NA |
| 1.5 | 3 | 0x00 00 00 00 | 00000 | 1 | 0 | Cell B | UAC_BarringInfo_Common (3,0x0100000'B) | 246 / 082 | No | NA |
| 1.6 | 3 | 0x00 00 00 00 | 00000 | 1 | 0 | Cell B | UAC_BarringInfo_PerPLMN (3,0x01000001'B) | 244 / 081 | No | NA |
| 1.7 | 7 | 0x00 00 00 00 | 00000 | 1 | 0 | Cell B | UAC_BarringInfo_Common( 7,0x0000000'B) | 244 / 081 | Yes | No |

##### 5.4.4.4.3	Procedure

Procedure/steps to be repeated for all sequences listed in table 5.4.3-1:

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | TT | SIB1 of Cell A is transmitted without uac-BarringInfo in SIB1. |  |  |  |
| 2 | UE | Power UE on | The UE performs a SIM initialisation. (includes reading of EFUAC-AIC and EFACC) | CR 1 | A.2/1 OR A.2/2 |
| 3 | UE > TT | REGISTRATION REQUEST is sent |  |  |  |
| 4 | TT > UE | Send REGISTRATION ACCEPT | The MPS indicator bit set to "Access identity 1 valid". For simplicity other signalling is not shown |  |  |
| 5 | UE | Power UE on |  |  |  |
| 6 | TT | Deactivates Cell A |  |  |  |
| 7 | TT | Activates Cell A or B with SIB1 as specified in the table 5.4.3-1 | See the column 'SIB1 of Cell A or B after the 2nd power up of the ME' for the SIB 1 and the column ' Cell A or B' to find the cell to be activated for this test step. |  |  |
| 8 | UE | Power UE on | Power on with a different USIM (e.g.: IMSI set to 24681685533963) The UE performs a SIM initialisation again. |  |  |

| If cell access is not allowed: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 9 | UE > TT | REGISTRATION REQUEST in not sent | See column ' Registration successful?' for the result | CR 2 through CR 5 |  |
| 10 | USER | End test sequence |  |  |  |

| If cell access is allowed: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 9 | UE > TT | Send REGISTRATION REQUEST | See column ' Registration successful?' for the result | CR 2 through CR 5 |  |
| 10 | TT > UE | Send REGISTRATION ACCEPT | MPS and MCS indicator bits are set to zero in 5GS network feature support IE. For simplicity other signalling is not shown |  |  |

| If MO Data call is not allowed: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 11 | UE | Attempt to set up MO Data call | To set up the MO Data call the MMI or EMMI is used | CR 2 through CR 5 |  |
| 12 | USER | End test sequence |  |  |  |

| If MO Data call is allowed: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 11 | UE > TT | Set up MO Data call (PDU SessionEstablishment) | To set up the MO Data call the MMI or EMMI is used | CR 2 through CR 5 |  |
| 12 | USER | End test sequence |  |  |  |

#### 5.4.4.5	Acceptance criteria

For the scenarios in table 5.4.4-1:

CR 1 is explicitly verified at step 2) by analysing the file READ commands for EFUAC-AIC and EFACC during the USIM-Terminal initialisation procedure.

CR 2, CR 3, CR 4, and CR 5 are verified:

- at steps 9) and 10) by analysing if the UE shall make a successful or unsuccessful Registration to the network in accordance with the result indicated in the table and

- at steps 11) and 12) by analysing if the UE shall make a successful or unsuccessful MO Data call in accordance with the result indicated in the table if the step is applicable.

### 5.4.5	Unified Access Control – Access Identity 2 – MCS indicated by USIM

#### 5.4.5.1	Definition and applicability

The purpose of Unified Access Control procedure is to perform access barring check for a 5GS access attempt associated with a given Access Category and one or more Access Identities upon request from upper layers or the RRC layer.

The 5G network shall be able to broadcast barring control information (i.e. a list of barring parameters associated with an Access Identity and an Access Category) in SIB1.

The EFUAC-AIC in the USIM contains the configuration information pertaining to access identity 2 allocated for high priority services MCS.

The UE shall be able to determine whether or not a particular new access attempt is allowed based on barring parameters that the UE receives from the broadcast barring control information and the configuration in the USIM.

When the NAS detects an access event, the NAS shall perform the mapping of the kind of request to one or more access identities and one access category and lower layers will perform access barring checks for that request based on the determined access identities and access category.

#### 5.4.5.2	Conformance requirement

CR 1	The UE shall read the access control value as part of the USIM-Terminal initialization procedure, and subsequently adopt this value.

Reference:

-	TS 31.102 [19], clause 5.1.1.2.

CR 2	Access Identities are configured at the UE as listed in TS 22.261 [36] Table 6.22.2.2-1. Access Categories are defined by the combination of conditions related to UE and the type of access attempt as listed in TS 22.261 [36] Table 6.22.2.3-1. One or more Access Identities and only one Access Category are selected and tested for an access attempt.

Reference:

-	TS 22.261 [36], clause 6.22.2

CR 3	Access Identity 2 is valid if the RPLMN is the HPLMN, EHPLMN or visited PLMN of the home country.

Reference:

-	TS 24.501 [25], clause 4.5.2

CR 4	The UE shall be able to determine whether or not a particular new access attempt is allowed based on uac-BarringInfo broadcast in SIB1.  Access Control check shall be performed as per the information received in uac-BarringInfoSetList.

Reference:

-	TS 38.331 [37], clauses 5.3.14

#### 5.4.5.3	Test purpose

The purpose of this test is to verify that:

1)	the ME reads the access control value from EFUAC-AIC and EFACC as part of the USIM-Terminal initialisation procedure, and subsequently adopts this value

2)	the UE maps the kind of request to one or more access identities and one access category and lower layers performs access barring checks for that request based on the determined access identities and access category;

3)	the UE determines whether or not a particular access attempt is allowed based on uac-BarringInfo broadcast in SIB1 and if the RPLMN is the HPLMN, EHPLMN or visited PLMN of the home country.

#### 5.4.5.4	Method of test

##### 5.4.5.4.1	Initial conditions

The values of the 5G-NR UICC as defined in clause 4.5.9 of the present document are used with EFIMSI as defined in clause 4.6.5 and the following exceptions:

EFUAC_AIC and EFACC

No Access Identity is configured in EFUAC_AIC and no Access Classes are configured in EFACC as given in table 5.4.5-1.

EFUST (USIM Service Table)

Logically:

| Service n°124 |  | Subscription identifier privacy support | not available |
| --- | --- | --- | --- |
| Service n°126 |  | UAC Access Identities support | available |

Coding:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary: | xxxx xx1x | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxxx xxxx | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | … | B16 |  |  |  |
|  | xxxx xxxx | xxxx xxxx | xx11 xxxx | ... | xx10 011x |  |  |  |

The defined UICC/USIM data is installed on the UE.

The TT (NG-SS) is configured to transmit on the BCCH with the following parameters:

Cell A:

-	TAI (MCC/MNC/TAC):		MCC, MNC: see tables 5.4.5-x, TAC: 000001.

-	CellIdentity:					000000001

For Table 5.4.5-1:

uac-BarringInfo in SIB1 should be set as in the table:

-	Refer to Annex A for the Methods UAC_BarringInfo_xxxxxx() in the table.

##### 5.4.5.4.2	Test sequence settings

Table 5.4.5-1

| TC Seq# | Access Category | USIM |  | SIB1 |  | REGISTRATION ACCEPT (5GS network feature support IE) |  | Result |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | EFUAC_AIC | EFACC (Byte 1 b8-b4) | uac-BarringInfo | PLMN-Identity (MCC/MNC) | MPS indicator Bit | MCS indicator Bit | Registration successful? | MO Data call successful? |
| 1.1 | 7 | 0x02 00 00 00 | 00000 | Not Present | 246 / 081 | 0 | 0 | Yes | Yes |
| 1.2 | 3 | 0x02 00 00 00 | 00000 | UAC_BarringInfo_Common (3,0x0100000'B) | 246 / 081 | 0 | 0 | No | NA |
| 1.3 | 3 | 0x02 00 00 00 | 00000 | UAC_BarringInfo_PerPLMN (3,0x0100000'B) | 246 / 081 | 0 | 0 | No | NA |
| 1.4 | 3 | 0x02 00 00 00 | 00000 | UAC_BarringInfo_Common (3,0x0100000'B) | 246 / 082 | 0 | 0 | No | NA |
| 1.5 | 3 | 0x02 00 00 00 | 00000 | UAC_BarringInfo_Common (3,0x1000000'B) | 246 / 081 | 0 | 0 | Yes | Yes |
| 1.6 | 3 | 0x03 00 00 00 | 00000 | UAC_BarringInfo_Common (3,0x1000000'B) | 246 / 081 | 0 | 0 | Yes | Yes |
| 1.7 | 7 | 0x02 00 00 00 | 00000 | UAC_BarringInfo_Common (7,0x0000000'B) | 246 / 081 | 0 | 0 | Yes | Yes |
| 1.8 | 3 | 0x02 00 00 00 | 00000 | UAC_BarringInfo_Common (3,0x0000000'B) | 244 / 081 | 0 | 0 | No | NA |
| 1.9 | 7 | 0x02 00 00 00 | 00000 | UAC_BarringInfo_Common (7,0x0100000'B) | 246 / 081 | 0 | 0 | Yes | No |
| 1.10 | 7 | 0x02 00 00 00 | 00000 | UAC_BarringInfo_PerPLMN (7,0x0100000'B) | 246 / 081 | 0 | 0 | Yes | No |
| 1.11 | 7 | 0x02 00 00 00 | 00000 | UAC_BarringInfo_Common (7,0x0100000'B) | 246 / 082 | 0 | 0 | Yes | No |
| 1.12 | 7 | 0x02 00 00 00 | 00000 | UAC_BarringInfo_Common (7,0x0000000'B) | 244 / 081 | 0 | 0 | Yes | No |
| 1.13 | 7 | 0x02 00 00 00 | 00000 | UAC_BarringInfo_Common (7,0x10000000'B) | 244 / 081 | 0 | 1 | Yes | Yes |
| 1.14 | 7 | 0x01 00 00 00 | 00000 | UAC_BarringInfo_Common (7,0x10000000'B) | 244 / 081 | 0 | 1 | Yes | Yes |
| 1.15 | 7 | 0x02 00 00 00 | 00000 | UAC_BarringInfo_Common2 (3,0x1000000'B, 7,0x0100000'B) | 246 / 081 | 0 | 0 | Yes | No |
| 1.16 | 3 | 0x02 00 00 00 | 00000 | UAC_BarringInfo_Common2 (7,0x0100000'B, 3,0x0000000'B) | 244 / 081 | 0 | 0 | No | NA |

##### 5.4.5.4.3	Procedure

Procedure/steps to be repeated for all sequences listed in table 5.4.5-1:

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | TT | SIB1 of Cell A is transmitted as defined in the initial conditions for tests from table 5.4.5-1 | Barring info is set as in table 5.4.5-1 See Annex A for the Methods UAC_BarringInfo_xxxxxx() in the table |  |  |
| 2 | UE | Power UE on | The UE performs a SIM initialisation. (includes reading of EFUAC-AIC and EFACC) | CR 1 | A.2/1 OR A.2/2 |

| If cell access is not allowed: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 3 | UE > TT | REGISTRATION REQUEST is not sent | See column 'Registration successful?' for the result | CR 2 CR 3 CR 4 |  |
| 4 | User | End test sequence |  |  |  |

| If cell access is allowed: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 3 | UE > TT | Send REGISTRATION REQUEST | See column 'Registration successful?' for the result | CR 2 CR 3 CR 4 |  |
| 4 | TT > UE | Send REGISTRATION ACCEPT | For simplicity other signalling is not shown |  |  |

| If MO Data call is not allowed: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 5 | UE | Attempt to set up MO Data call | To set up the MO Data call the MMI or EMMI is used | CR 2 CR 3 CR 4 |  |
| 6 | User | End test sequence |  |  |  |

| If MO Data call is allowed: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 5 | UE > TT | Set up MO Data call (PDU SessionEstablishment) | To set up the MO Data call the MMI or EMMI is used | CR 2 CR 3 CR 4 |  |
| 6 | User | End test sequence |  |  |  |

#### 5.4.5.5	Acceptance criteria

For the scenarios in table 5.4.5-1:

CR 1 is explicitly verified at step 2) by analysing the file READ commands for EFUAC-AIC and EFACC during the USIM-ME initialisation procedure.

CR 2, CR 3, and CR 4 are verified

- step 3) by analysing if the UE shall make a successful or unsuccessful registration to the network in accordance with the result indicated in the table and

- step 5) by analysing if the UE shall make a successful or unsuccessful MO Data call in accordance with the result indicated in the table if the step is applicable.

### 5.4.5A	Unified Access Control – Access Identity 2 – MCS indicated by USIM

#### 5.4.5A.1	Definition and applicability

The purpose of Unified Access Control procedure is to perform access barring check for a 5GS access attempt associated with a given Access Category and one or more Access Identities upon request from upper layers or the RRC layer.

The 5G network shall be able to broadcast barring control information (i.e. a list of barring parameters associated with an Access Identity and an Access Category) in SIB1.

The EFUAC-AIC in the USIM contains the configuration information pertaining to access identity 2 allocated for high priority services MCS.

The UE shall be able to determine whether or not a particular new access attempt is allowed based on barring parameters that the UE receives from the broadcast barring control information and the configuration in the USIM.

When the NAS detects an access event, the NAS shall perform the mapping of the kind of request to one or more access identities and one access category and lower layers will perform access barring checks for that request based on the determined access identities and access category.

If RRC state is RRC_INACTIVE and the resumption of the RRC connection is triggered due to an RNA Update RRC layer shall select Access Category as 8 and perform unified access control procedure in case there is no ongoing emergency service.

#### 5.4.5A.2	Conformance requirement

CR 1	The ME shall read the access control value as part of the USIM-Terminal initialization procedure, and subsequently adopt this value.

Reference:

-	TS 31.102 [19], clause 5.1.1.2.

CR 2	Access Identities are configured at the UE as listed in TS 22.261 [36] Table 6.22.2.2-1. Access Categories are defined by the combination of conditions related to UE and the type of access attempt as listed in TS 22.261 [36] Table 6.22.2.3-1. One or more Access Identities and only one Access Category are selected and tested for an access attempt.

Reference:

-	TS 22.261 [36], clause 6.22.2

CR 3	Access Identity 2 is valid if the RPLMN is the HPLMN, EHPLMN or visited PLMN of the home country.

Reference:

-	TS 24.501 [25], clause 4.5.2

CR 4	The UE shall be able to determine whether or not a particular new access attempt is allowed based on uac-BarringInfo broadcast in SIB1.  Access Control check shall be performed as per the information received in uac-BarringInfoSetList.

Reference:

-	TS 38.331 [37], clauses 5.3.14

CR 5	If the resumption of the RRC connection is triggered due to an RNA update and there is no ongoing emergency service RRC shall select '8' as the Access Category and perform the unified access control procedure.

Reference:

-	TS 38.331 [37], clauses 5.3.13.2

#### 5.4.5A.3	Test purpose

The purpose of this test is to verify that:

1)	the ME reads the access control value from EFUAC-AIC and EFACC as part of the USIM-Terminal initialisation procedure, and subsequently adopts this value;

2)	the UE maps the kind of request to one or more access identities and one access category and lower layers performs access barring checks for that request based on the determined access identities and access category;

3)	the UE determines whether or not a particular access attempt is allowed based on uac-BarringInfo broadcast in SIB1 and if the RPLMN is the HPLMN, EHPLMN or visited PLMN of the home country;

4)	the UE performs unified access control procedure if RNA Update procedure is triggered.

#### 5.4.5A.4	Method of test

##### 5.4.5A.4.1	Initial conditions

The values of the 5G-NR UICC as defined in clause 4.5.9 of the present document are used with EFIMSI as defined in clause 4.6.5 and the following exceptions:

EFUAC_AIC and EFACC

No Access Identities configured in EFUAC_AIC and no Access Classes configured in EFACC as also shown in table 5.4.5A-1.

EFUST (USIM Service Table)

Logically:

| Service n°124 |  | Subscription identifier privacy support | not available |
| --- | --- | --- | --- |
| Service n°126 |  | UAC Access Identities support | available |

Coding:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary: | xxxx xx1x | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxxx xxxx | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | … | B16 |  |  |  |
|  | xxxx xxxx | xxxx xxxx | xx11 xxxx | ... | xx10 011x |  |  |  |

The defined UICC/USIM data is installed on the UE.

The TT (NG-SS) is configured to transmit on the BCCH with the following parameters:

Cell A:

-	TAI (MCC/MNC/TAC):		MCC, MNC: see table 5.4.5A-1, TAC: 000001.

-	CellIdentity:					000000001

For Table 5.4.5A-1:

No uac-BarringInfo in SIB1.

Cell B:

Transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):		MCC, MNC: see table 5.4.5A-1, TAC: 000001.

-	CellIdentity:					000000002

For Table 5.4.5A-1:

uac-BarringInfo in SIB1 should be set as in the table:

-	Refer to Annex A for the Methods UAC_BarringInfo_xxxxxx() in the table.

##### 5.4.5A.4.2	Test sequence settings

Table 5.4.5A-1

| TC Seq# | Access Category | USIM |  | SIB1 |  | REGISTRATION ACCEPT (5GS network feature support IE) |  | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | EFUAC_AIC | EFACC (Byte 1, b8-b4) | uac-BarringInfo | PLMN-Identity (MCC/MNC) | MPS indicator Bit | MCS indicator Bit | RRCResume Request -rna-Update successful? |
| 2.1 | 8 | 0x02 00 00 00 | 00000 | Not Present | 246 / 081 | 0 | 0 | Yes |
| 2.2 | 8 | 0x02 00 00 00 | 00000 | UAC_BarringInfo_Common (8,0x0100000'B) | 246 / 081 | 0 | 0 | No |
| 2.3 | 8 | 0x02 00 00 00 | 00000 | UAC_BarringInfo_PerPLMN (8,0x0000000'B) | 246 / 081 | 0 | 0 | Yes |
| 2.4 | 8 | 0x02 00 00 00 | 00000 | UAC_BarringInfo_Common (8,0x0100000'B) | 244 / 081 | 0 | 1 | No |
| 2.5 | 8 | 0x02 00 00 00 | 00000 | UAC_BarringInfo_Common2 (7,0x0100000'B, 8,0x0000000'B) | 244 / 081 | 0 | 0 | No |

##### 5.4.5A.4.2	Procedure

Procedure/steps to be repeated for all sequences listed in table 5.4.5A-1:

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | TT | SIB1 of Cell A is transmitted as defined in the initial conditions and in table 5.4.5A-1 | No barring info is provided to Cell A |  |  |
| 2 | UE | Power UE on | The UE performs a SIM initialisation | CR 1 | A.2/1 OR A.2/2 |
| 3 | UE > TT | Send REGISTRATION REQUEST |  |  |  |
| 4 | TT > UE | Send REGISTRATION ACCEPT | The registration is successful |  |  |
| 5 | UE > TT | Set up MO Data call (PDU SessionEstablishment) | To set up the MO Data call the MMI or EMMI is used. The MO Data call is set up successfully |  |  |
| 6 | TT > UE | Send RRC RELEASE with suspendConfig in criticalExtensions (with the choice rrcRelease) | ran-NotificationAreaInfo in suspendConfig contains the cellList with cellIdentity of Cell A: - cellList { - plmn-Identity {mcc, mnc}, - -- see table 5.4.5A-1 for MCC/MNC - ran-AreaCells 000000001’B } |  |  |
| 7 | TT | Deactivate Cell A and activate Cell B |  |  |  |
| 8 | TT | SIB1 of Cell B is transmitted as defined in table 5.4.5A-1 | Barring info is as in the table. See Annex A for the Methods UAC_BarringInfo_xxxxxx() in the table |  |  |
| 9 | UE > TT | Initiate RRC RESUME REQUEST procedure with resumeCause set to rna-Update. | See the column ‘RRC RESUME REQUEST with resumeCause set to rna-Update successful?’ for the result | CR 2 CR 3 CR 4 CR 5 |  |

#### 5.4.5A.5	Acceptance criteria

For the scenarios in table 5.4.5A-1:

CR 1 is explicitly verified at step 2) by analysing the file READ commands for EFUAC-AIC and EFACC during the USIM-Terminal initialisation procedure.

CR 2, CR 3, CR 4, and CR 5 are verified at step 9) by analysing if the UE shall make a successful or unsuccessful RRC Resumption for RNA Update in accordance with the result indicated in the table.

### 5.4.6	Unified Access Control – Access Identity 2 – no MCS indication by USIM and SUPI is not changed

#### 5.4.6.1	Definition and applicability

The purpose of Unified Access Control procedure is to perform access barring check for a 5GS access attempt associated with a given Access Category and one or more Access Identities upon request from upper layers or the RRC layer.

The 5G network shall be able to broadcast barring control information (i.e. a list of barring parameters associated with an Access Identity and an Access Category) in SIB1.

When the UE is in the country of its HPLMN or in an EHPLMN (if the EHPLMN list is present), and the USIM file EFUAC_AIC does not indicate the UE is configured for access identity 2, the UE uses the MCS indicator bit of the 5GS network feature support IE in the REGISTRATION ACCEPT message to determine if access identity 2 is valid.

The UE shall not consider access identity 2 to be valid when the UE is not in the country of its HPLMN prior to receiving the MCS indicator bit of the 5GS network feature support IE in the REGISTRATION ACCEPT message being set to "Access identity 2 valid in RPLMN or equivalent PLMN".

The UE shall be able to determine whether or not a particular new access attempt is allowed based on barring parameters that the UE receives from the broadcast barring control information and the configuration in the USIM.

When the NAS detects an access event, the NAS shall perform the mapping of the kind of request to one or more access identities and one access category and lower layers will perform access barring checks for that request based on the determined access identities and access category.

#### 5.4.6.2	Conformance requirement

CR 1	The USIM file EFUAC_AIC does not indicate the UE is configured for access identity 2, the UE uses the MCS indicator bit of the 5GS network feature support IE in the REGISTRATION ACCEPT message to determine if access identity 2 is valid.

Reference:

-	TS 24.501 [25], clause 4.5.2.

CR 2	The UE shall not consider access identity 2 to be valid when the UE is not in the country of its HPLMN prior to receiving the MCS indicator bit of the 5GS network feature support IE in the REGISTRATION ACCEPT message being set to "Access identity 2 valid in RPLMN or equivalent PLMN".

Reference:

-	TS 24.501 [25], clause 4.5.2.

CR 3	The MCS indicator is stored together with a PLMN identity of the PLMN that provided it, and is valid in that RPLMN or equivalent PLMN. The MCS indicator can only be used if the SUPI from the USIM matches the SUPI stored in the non-volatile memory of the ME, else the UE shall delete the MCS indicator.

Reference:

-	TS 24.501 [25], Annex C.

CR 4	Access Identity 2 is valid if the RPLMN is the HPLMN, EHPLMN or visited PLMN of the home country.

Reference:

-	TS 24.501 [25], clause 4.5.2

CR 5	Upon receiving a REGISTRATION ACCEPT message with the MCS indicator bit set to "Access identity 2 valid", the UE shall act as a UE with access identity 2 configured for MCS in all NG-RAN of the registered PLMN and its equivalent PLMNs. The MCS indicator bit in the 5GS network feature support IE provided in the REGISTRATION ACCEPT message is valid until the UE receives a REGISTRATION ACCEPT message with the MCS indicator bit set to "Access identity 2 not valid" or until the UE selects a non-equivalent PLMN. Access identity 2 is only applicable while the UE is in N1 mode.

Reference:

-	TS 24.501 [25], clause 5.5.1.3.4

CR 6	The UE shall be able to determine whether or not a particular new access attempt is allowed based on uac-BarringInfo broadcast in SIB1. Access Control check shall be performed as per the information received in uac-BarringInfoSetList.

Reference:

-	TS 38.331 [37], clauses 5.3.14

#### 5.4.6.3	Test purpose

The purpose of this test is to verify that:

1)	if the RPLMN is the HPLMN, EHPLMN or visited PLMN of the home country and the USIM file EFUAC_AIC does not indicate the UE is configured for access identity 2 and the MCS indicator bit of the 5GS network feature support IE in the REGISTRATION ACCEPT message is received, UE considers access identity as 2 for access barring;

2)	the MCS indicator is stored together with a PLMN identity of the PLMN that provided it, and the MCS indicator can only be used if the SUPI from the USIM matches the SUPI stored in the non-volatile memory of the ME;

3)	the UE maps the kind of request to one or more access identities and one access category and lower layers performs access barring checks for that request based on the determined access identities and access category;

4)	the UE determines whether or not a particular access attempt is allowed based on uac-BarringInfo broadcast in SIB1 and if the RPLMN is the HPLMN, EHPLMN or visited PLMN of the home country;

5)	a UE operated with a USIM where the file EFUAC_AIC does not indicate access identity 2, after a change of the REGISTRATION ACCEPT message with the MCS indicator bit set to "Access identity 2 not valid", or when the UE has selected a non-equivalent PLMN, the UE is acting as if it is configured for access identity 0.

#### 5.4.6.4	Method of test

##### 5.4.6.4.1	Initial conditions

The values of the 5G-NR UICC as defined in clause 4.5.9 of the present document are used with EFIMSI as defined in clause 4.6.5 and the following exceptions:

EFUAC_AIC and EFACC

No Access Identities configured in EFUAC_AIC and no Access Classes configured in EFACC as also shown in table 5.4.6-1.

EFUST (USIM Service Table)

Logically:

| Service n°124 |  | Subscription identifier privacy support | not available |
| --- | --- | --- | --- |
| Service n°126 |  | UAC Access Identities support | available |

Coding:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary: | xxxx xx1x | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxxx xxxx | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | … | B16 |  |  |  |
|  | xxxx xxxx | xxxx xxxx | xx11 xxxx | ... | xx10 011x |  |  |  |

The defined UICC/USIM data is installed on the UE.

The TT (NG-SS) is configured to transmit on the BCCH with the following parameters:

Cell A:

-	MCC, MNC:	246/081

-	TAC:	=		000001

-	CellIdentity:	000000001

For Table 5.4.6-1:

After turning on the cell for the 2nd time, uac-BarringInfo in SIB1 should be set as in the table:

Refer to Annex A for the Methods UAC_BarringInfo_xxxxxx() in the table.

Cell B:

-	TAI (MCC/MNC/TAC):		MCC, MNC: see table 5.4.6-1, TAC: 000002.

-	CellIdentity:					000000002

For Table 5.4.6-1:

uac-BarringInfo in SIB1 should be set as in the table:

Refer to Annex A for the Methods UAC_BarringInfo_xxxxxx() in the table.

##### 5.4.6.4.2	Test sequence settings

Table 5.4.6-1

| TC Seq# | Access Category | USIM |  | REGISTRATION ACCEPT (5GS network feature support IE) on Cell A at 1st power up of the ME |  | SIB1 of Cell A or B after the 2nd power up of the ME |  |  | Result |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | EFUAC_AIC | EFACC (Byte 1 b8-b4) | MPS indicator Bit | MCS indicator Bit | Cell A or B | uac-BarringInfo | PLMN-Identity (MCC/MNC) | Registration successful? | MO Data call successful? |
| 1.1 | 3 | 0x00 00 00 00 | 00000 | 1 | 0 | Cell A | Not Present | 246 / 081 | Yes | Yes |
| 1.2 | 3 | 0x00 00 00 00 | 00000 | 0 | 1 | Cell A | UAC_BarringInfo_Common (3,0x0100000'B) | 246 / 081 | No | NA |
| 1.3 | 7 | 0x00 00 00 00 | 00000 | 0 | 1 | Cell A | UAC_BarringInfo_Common (7,0x1000000'B) | 246 / 081 | Yes | No |
| 1.4 | 3 | 0x00 00 00 00 | 00000 | 0 | 1 | Cell A | UAC_BarringInfo_Common (3,0x1000000'B) | 246 / 081 | Yes | Yes |
| 1.5 | 3 | 0x00 00 00 00 | 00000 | 0 | 1 | Cell B | UAC_BarringInfo_Common (3,0x1000000'B) | 246 / 082 | No | NA |
| 1.6 | 3 | 0x00 00 00 00 | 00000 | 0 | 1 | Cell B | UAC_BarringInfo_PerPLMN (3,0x1000000'B) | 244 / 081 | No | NA |
| 1.7 | 7 | 0x00 00 00 00 | 00000 | 0 | 1 | Cell B | UAC_BarringInfo_Common (7,0x0000000'B) | 244 / 081 | Yes | No |

##### 5.4.6.4.3	Procedure

Procedure/steps to be repeated for all sequences listed in table 5.4.6-1:

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | TT | SIB1 of Cell A is transmitted without uac-BarringInfo in SIB1. |  |  |  |
| 2 | UE | Power UE on | The UE performs a SIM initialisation. (includes reading of EFUAC-AIC and EFACC) | CR 1 | A.2/1 OR A.2/2 |
| 3 | UE > TT | REGISTRATION REQUEST is sent |  |  |  |
| 4 | TT > UE | Send REGISTRATION ACCEPT | The MPS indicator bit set to "Access identity 1 valid". For simplicity other signalling is not shown |  |  |
| 5 | UE | Power UE off |  |  |  |
| 6 | TT | Deactivate Cell A |  |  |  |
| 7 | TT | Activate Cell A or B with SIB1 as specified in the table 5.4.6-1 | See the column 'SIB1 of Cell A or B after the 2nd power up of the ME' for the SIB 1 and the column 'Cell A or B' to find the cell to be activated for this test step. |  |  |
| 8 | UE | Power UE on | The UE performs a SIM initialisation again. |  |  |
| If cell access is not allowed: |  |  |  |  |  |
| 9 | UE > TT | REGISTRATION REQUEST in not sent | See column 'Registration successful?' for the result | CR 2 through CR 6 |  |
| 10 | USER | End test sequence |  |  |  |
| If cell access is allowed: |  |  |  |  |  |
| 9 | UE > TT | Send REGISTRATION REQUEST | See column 'Registration successful?' for the result | CR 2 through CR 6 |  |
| 10 | TT > UE | Send REGISTRATION ACCEPT | MPS and MCS indicator bits are set to zero in 5GS network feature support IE. For simplicity other signalling is not shown |  |  |
| If MO Data call is not allowed: |  |  |  |  |  |
| 11 | UE | Attempt to set up MO Data call | To set up the MO Data call the MMI or EMMI is used | CR 2 through CR 6 |  |
| 12 | USER | End test sequence |  |  |  |
| If MO Data call is allowed: |  |  |  |  |  |
| 11 | UE > TT | Set up MO Data call (PDU SessionEstablishment) | To set up the MO Data call the MMI or EMMI is used | CR 2 through CR 6 |  |
| 12 | USER | End test sequence |  |  |  |

##### 5.4.6.4.3	Acceptance criteria

For the scenarios in table 5.4.6-1:

CR 1 is explicitly verified at step 2) by analysing the file READ commands for EFUAC-AIC and EFACC during the USIM-Terminal initialisation procedure.

CR 2, CR 3, CR 4, CR 5, and CR 6 are verified at:

- steps 9) and 10) by analysing if the UE shall make a successful or unsuccessful Registration to the network in accordance with the result indicated in the table and

- steps 11) and 12) by analysing if the UE shall make a successful or unsuccessful MO Data call in accordance with the result indicated in the table if the step is applicable.

### 5.4.7	Unified Access Control – Access Identity 2 – no MCS indication by USIM and SUPI is changed

#### 5.4.7.1	Definition and applicability

The purpose of Unified Access Control procedure is to perform access barring check for a 5GS access attempt associated with a given Access Category and one or more Access Identities upon request from upper layers or the RRC layer.

The 5G network shall be able to broadcast barring control information (i.e. a list of barring parameters associated with an Access Identity and an Access Category) in SIB1.

When the USIM file EFUAC_AIC does not indicate the UE is configured for access identity 2, the UE uses the MCS indicator bit of the 5GS network feature support IE in the REGISTRATION ACCEPT message to determine if access identity 2 is valid.

The MCS indicator is stored together with a PLMN identity of the PLMN that provided it, and is valid in that RPLMN or equivalent PLMN. The MCS indicator can only be used if the SUPI from the USIM matches the SUPI stored in the non-volatile memory of the ME, else the UE shall delete the MCS indicator.

The UE shall be able to determine whether or not a particular new access attempt is allowed based on barring parameters that the UE receives from the broadcast barring control information and the configuration in the USIM.

When the NAS detects an access event, the NAS shall perform the mapping of the kind of request to one or more access identities and one access category and lower layers will perform access barring checks for that request based on the determined access identities and access category.

#### 5.4.7.2	Conformance requirement

CR 1	When the UE is in the country of its HPLMN, and the USIM file EFUAC_AIC does not indicate the UE is configured for access identity 2, the UE uses the MCS indicator bit of the 5GS network feature support IE in the REGISTRATION ACCEPT message to determine if access identity 2 is valid.

Reference:

-	TS 24.501 [25], clause 4.5.2.

CR 2	The UE shall not consider access identity 2 to be valid when the UE is not in the country of its HPLMN prior to receiving the MCS indicator bit of the 5GS network feature support IE in the REGISTRATION ACCEPT message being set to "Access identity 2 valid in RPLMN or equivalent PLMN".

Reference:

-	TS 24.501 [25], clause 4.5.2.

CR 3	The UE shall consider access identity 2 to be valid when the network informs the UE that the use of access identity 2 is valid in the RPLMN or equivalent PLMN by setting the MCS indicator bit of the 5GS network feature support IE to "Access identity 2 valid", in the REGISTRATION ACCEPT message, the UE shall act as a UE with access identity 2 configured for MCS in all NG-RAN of the registered PLMN and its equivalent PLMNs, until the UE receives a REGISTRATION ACCEPT message with the MCS indicator bit set to "Access identity 2 not valid" or until the UE selects a non-equivalent PLMN. Access identity 2 is only applicable while the UE is in N1 mode.

Reference:

-	TS 24.501 [25], clause 5.5.1.2.4.

CR 4	The MCS indicator is stored together with a PLMN identity of the PLMN that provided it, and is valid in that RPLMN or equivalent PLMN. The MCS indicator can only be used if the SUPI from the USIM matches the SUPI stored in the non-volatile memory of the ME, else the UE shall delete the MCS indicator.

Reference:

-	TS 24.501 [25], Annex C.

CR 5	The UE shall be able to determine whether or not a particular new access attempt is allowed based on uac-BarringInfo broadcast in SIB1. Access Control check shall be performed as per the information received in uac-BarringInfoSetList.

Reference:

-	TS 38.331 [37], clauses 5.3.14

#### 5.4.7.3	Test purpose

The purpose of this test is to verify that:

1)	if the RPLMN is the HPLMN, EHPLMN or visited PLMN of the home country and the USIM file EFUAC_AIC does not indicate the UE is configured for access identity 2 and the MCS indicator bit of the 5GS network feature support IE in the REGISTRATION ACCEPT message is received, UE considers access identity as 2 for access barring.

2)	the MCS indicator bit in the 5GS network feature support IE provided in the REGISTRATION ACCEPT message is valid until the UE receives a REGISTRATION ACCEPT message with the MCS indicator bit set to "Access identity 2 not valid" or until the UE selects a non-equivalent PLMN.

3)	the MCS indicator is stored together with a PLMN identity of the PLMN that provided it, and the MCS indicator can only be used if the SUPI from the USIM matches the SUPI stored in the non-volatile memory of the ME.

4)	the UE maps the kind of request to one or more access identities and one access category and lower layers performs access barring checks for that request based on the determined access identities and access category.

5)	a UE operated with a USIM where the file EFUAC_AIC does not indicate access identity 2, after a change of the REGISTRATION ACCEPT message with the MCS indicator bit set to "Access identity 2 not valid", or when the UE has selected a non-equivalent PLMN, the UE is acting as if it is configured for access identity 0.

#### 5.4.7.4	Method of test

##### 5.4.7.4.1	Initial conditions

The values of the 5G-NR UICC as defined in clause 4.5.9 of the present document are used with EFIMSI as defined in clause 4.6.5 and the following exceptions:

EFUAC_AIC and EFACC

No Access Identities configured in EFUAC_AIC and no Access Classes configured in EFACC as also shown in table 5.4.7-1.

EFUST (USIM Service Table)

Logically:

| Service n°124 |  | Subscription identifier privacy support | not available |
| --- | --- | --- | --- |
| Service n°126 |  | UAC Access Identities support | available |

Coding:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary: | xxxx xx1x | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxxx xxxx | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | … | B16 |  |  |  |
|  | xxxx xxxx | xxxx xxxx | xx11 xxxx | ... | xx10 011x |  |  |  |

The defined UICC/USIM data is installed on the UE.

The TT (NG-SS) is configured to transmit on the BCCH with the following parameters:

Cell A:

-	TAI (MCC/MNC/TAC):		246/081/000001

-	CellIdentity:					000000001

For Table 5.4.7-1:

After turning on the cell for the 2nd time, uac-BarringInfo in SIB1 should be set as in the table:

-	Refer to Annex A for the Methods UAC_BarringInfo_xxxxxx() in the table.

Cell B:

-	TAI (MCC/MNC/TAC): 	MCC, MNC: see table 5.4.7-1, TAC: 000002.

-	CellIdentity:					000000002

For Table 5.4.7-1:

uac-BarringInfo in SIB1 should be set as in the table:

##### 5.4.7.4.2	Test sequence settings

Table 5.4.7-1

| TC Seq# | Access Category | USIM |  | REGISTRATION ACCEPT (5GS network feature support IE) on Cell A at 1st power up of the ME |  | SIB1 of Cell A or B after the 2nd power up of the ME |  |  | Result |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | EFUAC_AIC | EFACC (Byte 1 b8-b4) | MPS indicator Bit | MCS indicator Bit | Cell A or B | uac-BarringInfo | PLMN-Identity (MCC/MNC) | Registration successful? | MO Data call successful? |
| 1.1 | 3 | 0x00 00 00 00 | 00000 | 0 | 1 | Cell A | Not Present | 246 / 081 | Yes | Yes |
| 1.2 | 3 | 0x00 00 00 00 | 00000 | 0 | 1 | Cell A | UAC_BarringInfo_Common (3,0x1000000'B) | 246 / 081 | No | NA |
| 1.3 | 7 | 0x00 00 00 00 | 00000 | 0 | 1 | Cell A | UAC_BarringInfo_Common (7,0x1000000'B) | 246 / 081 | Yes | No |
| 1.4 | 3 | 0x00 00 00 00 | 00000 | 0 | 1 | Cell A | UAC_BarringInfo_Common (3,0x0000000'B) | 246 / 081 | No | NA |
| 1.5 | 3 | 0x00 00 00 00 | 00000 | 0 | 1 | Cell B | UAC_BarringInfo_Common (3,0x1000000'B) | 246 / 082 | No | NA |
| 1.6 | 3 | 0x00 00 00 00 | 00000 | 0 | 1 | Cell B | UAC_BarringInfo_PerPLMN (3,0x1000000'B) | 244 / 081 | No | NA |
| 1.7 | 7 | 0x00 00 00 00 | 00000 | 0 | 1 | Cell B | UAC_BarringInfo_Common (7,0x0000000'B) | 244 / 081 | Yes | No |

##### 5.4.7.4.3	Procedure

Procedure/steps to be repeated for all sequences listed in table 5.4.7-1:

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | TT | SIB1 of Cell A is transmitted without uac-BarringInfo in SIB1. |  |  |  |
| 2 | UE | Power UE on | The UE performs a SIM initialisation. (includes reading of EFUAC-AIC and EFACC) | CR 1 | A.2/1 OR A.2/2 |
| 3 | UE > TT | REGISTRATION REQUEST is sent |  |  |  |
| 4 | TT > UE | Send REGISTRATION ACCEPT | The MPS indicator bit set to "Access identity 2 valid". For simplicity other signalling is not shown |  |  |
| 5 | UE | Power UE on |  |  |  |
| 6 | TT | Deactivates Cell A |  |  |  |
| 7 | TT | Activates Cell A or B with SIB1 as specified in the table 5.4.7-1 | See the column 'SIB1 of Cell A or B after the 2nd power up of the ME' for the SIB 1 and the column 'Cell A or B' to find the cell to be activated for this test step. |  |  |
| 8 | UE | Power UE on | Power on with a different USIM (e.g.: IMSI set to 24681685533963) The UE performs a SIM initialisation again. |  |  |
| If cell access is not allowed: |  |  |  |  |  |
| 9 | UE > TT | REGISTRATION REQUEST in not sent | See column 'Registration successful?' for the result | CR 2 through CR 5 |  |
| 10 | USER | End test sequence |  |  |  |
| If cell access is allowed: |  |  |  |  |  |
| 9 | UE > TT | Send REGISTRATION REQUEST | See column 'Registration successful?' for the result | CR 2 through CR 5 |  |
| 10 | TT > UE | Send REGISTRATION ACCEPT | MPS and MCS indicator bits are set to zero in 5GS network feature support IE. For simplicity other signalling is not shown |  |  |
| If MO Data call is not allowed: |  |  |  |  |  |
| 11 | UE | Attempt to set up MO Data call | To set up the MO Data call the MMI or EMMI is used | CR 2 through CR 5 |  |
| 12 | USER | End test sequence |  |  |  |
| If MO Data call is allowed: |  |  |  |  |  |
| 11 | UE > TT | Set up MO Data call (PDU SessionEstablishment) | To set up the MO Data call the MMI or EMMI is used | CR 2 through CR 5 |  |
| 12 | USER | End test sequence |  |  |  |

#### 5.4.7.5	Acceptance criteria

For the scenarios in table 5.4.7-1:

CR 1 is explicitly verified at step 2) by analysing the file READ commands for EFUAC-AIC and EFACC during the USIM-Terminal initialisation procedure.

CR 2, CR 3, CR 4, and CR 5 are verified at:

- steps 9) and 10) by analysing if the UE shall make a successful or unsuccessful Registration to the network in accordance with the result indicated in the table and

- steps 11) and 12) by analysing if the UE shall make a successful or unsuccessful MO Data call in accordance with the result indicated in the table if the step is applicable.

### 5.4.8	Unified Access Control – Access Identities 11 and 15 indicated by USIM

#### 5.4.8.1	Definition and applicability

The purpose of Unified Access Control procedure is to perform access barring check for a 5GS access attempt associated with a given Access Category and one or more Access Identities upon request from upper layers or the RRC layer.

The 5G network shall be able to broadcast barring control information (i.e. a list of barring parameters associated with an Access Identity and an Access Category) in SIB1.

The EFACC in the USIM contains the configuration information pertaining to access identities 11 and 15.

The UE shall be able to determine whether or not a particular new access attempt is allowed based on barring parameters that the UE receives from the broadcast barring control information and the configuration in the USIM.

When the NAS detects an access event, the NAS shall perform the mapping of the kind of request to one or more access identities and one access category and lower layers will perform access barring checks for that request based on the determined access identities and access category.

#### 5.4.8.2	Conformance requirement

CR 1	The ME shall read the access control value as part of the USIM-Terminal initialization procedure, and subsequently adopt this value.

Reference:

-	3GPP TS 31.102 [19], clause 5.1.1.2.

CR 2	Access Identities are configured at the UE as listed in 3GPP TS 22.261 [43] Table 6.22.2.2-1. Access Categories are defined by the combination of conditions related to UE and the type of access attempt as listed in 3GPP TS 22.261 [43] Table 6.22.2.3-1. One or more Access Identities and only one Access Category are selected and tested for an access attempt.

Reference:

-	3GPP TS 22.261 [36], clause 6.22.2.

CR 3	Access Identity 11 and 15 are valid in HPLMN (if the EHPLMN list is not present or is empty) or EHPLMN (if the EHPLMN list is present).

Reference:

-	3GPP TS 24.501 [25], clause 4.5.2.

CR 4	The UE shall be able to determine whether or not a particular new access attempt is allowed based on uac-BarringInfo broadcast in SIB1.  Access Control check shall be performed as per the information received in uac-BarringInfoSetList.

Reference:

-	3GPP TS 38.331 [37], clauses 5.3.14.

#### 5.4.8.3	Test purpose

The purpose of this test is to verify that:

1)	the ME reads the access control value from EFUAC-AIC and EFACC as part of the USIM-Terminal initialisation procedure, and subsequently adopts this value.

2)	the UE maps the kind of request to one or more access identities and one access category and lower layers performs access barring checks for that request based on the determined access identities and access category.

3)	the UE determines whether or not a particular access attempt is allowed based on uac-BarringInfo broadcast in SIB1 and if the RPLMN is the HPLMN, EHPLMN or visited PLMN of the home country.

#### 5.4.8.4	Method of test

##### 5.4.8.4.1	Initial conditions

The values of the 5G-NR UICC as defined in clause 4.5.9 of the present document are used with EFIMSI as defined in clause 4.6.5 and the following exceptions:

EFUAC_AIC and EFACC

No Access Identities configured in EFUAC_AIC and no Access Classes configured in EFACC as also shown in table 5.4.8-1.

EFUST (USIM Service Table)

Logically:

| Service n°126 |  | UAC Access Identities support | available |
| --- | --- | --- | --- |

Coding:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary: | xxxx xx1x | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxxx xxxx | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | … | B16 | B17 |  |  |
|  | xxxx xxxx | xxxx xxxx | xx11 xxxx | ... | xx10 11xx | 0000 0xxx |  |  |

The defined UICC/USIM data is installed on the UE.

The TT (NG-SS) is configured to transmit on the BCCH with the following parameters:

Cell A:

-	TAI (MCC/MNC/TAC):	MCC, MNC: see tables 5.4.8-x, TAC: 000001.

-	CellIdentity:	000000001

For Table 5.4.8-1:

uac-BarringInfo in SIB1 should be set as in the table:

-	Refer to Annex A for the Methods UAC_BarringInfo_xxxxxx() in the table.

##### 5.4.8.4.2	Test sequence settings

Table 5.4.8-1

| TC Seq# | Access Category | USIM |  |  | SIB1 |  | Result |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | EFUAC_AIC | EFACC (Byte 1 b8-b4) | Access Identies | uac-BarringInfo | PLMN-Identity (MCC/MNC) | Registration successful? | MO Data call successful? |
| 1.1 | 7 | 0x00 00 00 00 | 00001 | 11 | Not Present | 246 / 081 | Yes | Yes |
| 1.2 | 3 | 0x00 00 00 00 | 00001 | 11 | UAC_BarringInfo_Common (3,0x0010000'B) | 246 / 081 | No | NA |
| 1.3 | 3 | 0x00 00 00 00 | 00001 | 11 | UAC_BarringInfo_PerPLMN (3,0x0010000'B) | 246 / 081 | No | NA |
| 1.4 | 3 | 0x00 00 00 00 | 00001 | 11 | UAC_BarringInfo_Common (3,0x0000000'B) | 246 / 082 | No | NA |
| 1.5 | 3 | 0x00 00 00 00 | 00001 | 11 | UAC_BarringInfo_Common (3,0x0001000'B) | 246 / 081 | Yes | Yes |
| 1.6 | 3 | 0x00 00 00 00 | 10001 | 11, 15 | UAC_BarringInfo_Common (3,0x0000001'B) | 246 / 081 | Yes | Yes |
| 1.7 | 7 | 0x00 00 00 00 | 00001 | 11 | UAC_BarringInfo_Common (7,0x0010000'B) | 246 / 081 | Yes | No |
| 1.8 | 7 | 0x00 00 00 00 | 00001 | 11 | UAC_BarringInfo_Common (7,0x0000000'B) | 246 / 082 | Yes | No |
| 1.9 | 7 | 0x00 00 00 00 | 00001 | 11 | UAC_BarringInfo_Common (7,0x0000000'B) | 244 / 081 | Yes | No |
| 1.10 | 7 | 0x00 00 00 00 | 00001 | 11 | UAC_BarringInfo_Common2 (3,0x0001000'B, 7,0x0010000'B) | 246 / 081 | Yes | No |
| 1.11 | 3 | 0x00 00 00 00 | 10000 | 15 | UAC_BarringInfo_Common (3,0x0000001'B) | 246 / 081 | No | NA |
| 1.12 | 3 | 0x00 00 00 00 | 10000 | 15 | UAC_BarringInfo_PerPLMN (3,0x0000001'B) | 246 / 081 | No | NA |
| 1.13 | 3 | 0x00 00 00 00 | 10000 | 15 | UAC_BarringInfo_Common (3,0x0000000'B) | 246 / 082 | No | NA |
| 1.14 | 3 | 0x00 00 00 00 | 10000 | 15 | UAC_BarringInfo_Common (3,0x0010000'B) | 246 / 081 | Yes | Yes |
| 1.15 | 7 | 0x00 00 00 00 | 10000 | 15 | UAC_BarringInfo_Common (7,0x0000001'B) | 246 / 081 | Yes | No |
| 1.16 | 7 | 0x00 00 00 00 | 10000 | 15 | UAC_BarringInfo_Common (7,0x0000000'B) | 246 / 082 | Yes | No |
| 1.17 | 7 | 0x00 00 00 00 | 10000 | 15 | UAC_BarringInfo_Common (7,0x0000000'B) | 244 / 081 | Yes | No |
| 1.18 | 7 | 0x00 00 00 00 | 10000 | 15 | UAC_BarringInfo_Common2 (3,0x0001000'B, 7,0x0000001'B) | 246 / 081 | Yes | No |

##### 5.4.8.4.3	Procedure

Procedure/steps to be repeated for all sequences listed in table 5.4.8-1:

| Step | Direction | Action | Comment | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | TT | SIB1 of Cell A is transmitted as defined in the initial conditions for tests from table 5.4.8-1 | Barring info is set as in table 5.4.8-1 See Annex A for the Methods UAC_BarringInfo_xxxxxx() in the table |  |  |
| 2 | UE | Power UE on | The UE performs a SIM initialisation. (includes reading of EFUAC-AIC and EFACC) | CR 1 | A.2/1 OR A.2/2 |

| If cell access is not allowed: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 3 | UE > TT | REGISTRATION REQUEST is not sent | See column ‘Registration successful?’ for the result | CR 2 CR 3 CR 4 |  |
| 4 | Tester | End test sequence |  |  |  |

| If cell access is allowed: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 3 | UE > TT | Send REGISTRATION REQUEST | See column ‘Registration successful?’ for the result | CR 2 CR 3 CR 4 |  |
| 4 | TT > UE | Send REGISTRATION ACCEPT | For simplicity other signalling is not shown |  |  |

| If MO Data call is not allowed: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 5 | UE | Attempt to set up MO Data call | To set up the MO Data call the MMI or EMMI is used | CR 2 CR 3 CR 4 |  |
| 6 | Tester | End test sequence |  |  |  |

| If MO Data call is allowed: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 5 | UE > TT | Set up MO Data call (PDU SessionEstablishment) | To set up the MO Data call the MMI or EMMI is used | CR 2 CR 3 CR 4 |  |
| 6 | Tester | End test sequence |  |  |  |

#### 5.4.8.5	Acceptance criteria

For the scenarios in table 5.4.8-1:

CR 1 is explicitly verified at step 2) by analysing the file READ commands for EFUAC-AIC and EFACC during the USIM-ME initialisation procedure.

CR 2, CR 3, and CR 4 are verified

- at step 3) by analysing if the UE shall make a successful or unsuccessful registration to the network in accordance with the result indicated in the table and

- at step 5) by analysing if the UE shall make a successful or unsuccessful MO Data call in accordance with the result indicated in the table if the step is applicable.

### 5.4.8A	Unified Access Control – Access Identities 11 and 15 indicated by USIM, Access Category 8

#### 5.4.8A.1	Definition and applicability

The purpose of Unified Access Control procedure is to perform access barring check for a 5GS access attempt associated with a given Access Category and one or more Access Identities upon request from upper layers or the RRC layer.

The 5G network shall be able to broadcast barring control information (i.e. a list of barring parameters associated with an Access Identity and an Access Category) in SIB1.

The EFACC in the USIM contains the configuration information pertaining to access identities 11 and 15.

The UE shall be able to determine whether or not a particular new access attempt is allowed based on barring parameters that the UE receives from the broadcast barring control information and the configuration in the USIM.

When the NAS detects an access event, the NAS shall perform the mapping of the kind of request to one or more access identities and one access category and lower layers will perform access barring checks for that request based on the determined access identities and access category.

If RRC state is RRC_INACTIVE and the resumption of the RRC connection is triggered due to an RNA Update RRC layer shall select Access Category as 8 and perform unified access control procedure in case there is no ongoing emergency service.

#### 5.4.8A.2	Conformance requirement

CR 1	The ME shall read the access control value as part of the USIM-Terminal initialization procedure, and subsequently adopt this value.

Reference:

-	3GPP TS 31.102 [19], clause 5.1.1.2.

CR 2	Access Identities are configured at the UE as listed in 3GPP TS 22.261 [36] Table 6.22.2.2-1. Access Categories are defined by the combination of conditions related to UE and the type of access attempt as listed in 3GPP TS 22.261 [36] Table 6.22.2.3-1. One or more Access Identities and only one Access Category are selected and tested for an access attempt.

Reference:

-	3GPP TS 22.261 [36], clause 6.22.2.

CR 3	Access Identity 11 and 15 are valid in HPLMN (if the EHPLMN list is not present or is empty) or EHPLMN (if the EHPLMN list is present).

Reference:

-	3GPP TS 24.501 [25], clause 4.5.2.

CR 4	The UE shall be able to determine whether or not a particular new access attempt is allowed based on uac-BarringInfo broadcast in SIB1.  Access Control check shall be performed as per the information received in uac-BarringInfoSetList.

Reference:

-	3GPP TS 38.331 [37], clauses 5.3.14.

CR 5	If the resumption of the RRC connection is triggered due to an RNA update and there is no ongoing emergency service RRC shall select '8' as the Access Category and perform the unified access control procedure.

Reference:

-	3GPP TS 38.331 [37], clauses 5.3.13.2.

#### 5.4.8A.3	Test purpose

The purpose of this test is to verify that:

1)	the ME reads the access control value from EFUAC-AIC and EFACC as part of the USIM-Terminal initialisation procedure, and subsequently adopts this value.

2)	the UE maps the kind of request to one or more access identities and one access category and lower layers performs access barring checks for that request based on the determined access identities and access category.

3)	the UE determines whether or not a particular access attempt is allowed based on uac-BarringInfo broadcast in SIB1 and if the RPLMN is the HPLMN, EHPLMN or visited PLMN of the home country.

4)	the UE performs unified access control procedure if RNA Update procedure is triggered.

#### 5.4.8A.4	Method of test

##### 5.4.8A.4.1	Initial conditions

The values of the 5G-NR UICC as defined in clause 4.5.9 of the present document are used with EFIMSI as defined in clause 4.6.5 and the following exceptions:

EFUAC_AIC and EFACC

No Access Identities configured in EFUAC_AIC and no Access Classes configured in EFACC as also shown in table 5.4.8A-1.

EFUST (USIM Service Table)

Logically:

| Service n°126 |  | UAC Access Identities support | available |
| --- | --- | --- | --- |

Coding:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary: | xxxx xx1x | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxxx xxxx | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | … | B16 | B17 |  |  |
|  | xxxx xxxx | xxxx xxxx | xx11 xxxx | ... | xx10 11xx | 0000 0xxx |  |  |

The defined UICC/USIM data is installed on the UE.

The TT (NG-SS) is configured to transmit on the BCCH with the following parameters:

Cell A:

-	TAI (MCC/MNC/TAC):	MCC, MNC: see table 5.4.8A-1, TAC: 000001.

-	CellIdentity:	000000001

For Table 5.4.8A-1:

No uac-BarringInfo in SIB1.

Cell B:

Transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):	MCC, MNC: see table 5.4.8A-1, TAC: 000001.

-	CellIdentity:	000000002

For Table 5.4.8A-1:

uac-BarringInfo in SIB1 should be set as in the table:

-	Refer to Annex A for the Methods UAC_BarringInfo_xxxxxx() in the table.

##### 5.4.8A.4.2	Test sequence settings

Table 5.4.8A-1

| TC Seq# | Access Category | USIM |  |  | SIB1 |  | Result |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | EFUAC_AIC | EFACC (Byte 1, b8-b4) | Access identity | uac-BarringInfo | PLMN-Identity (MCC/MNC) | RRCResumeRequest with resumeCause set to rna-Update successful? |
| 2.1 | 8 | 0x00 00 00 00 | 00001 | 11 | Not Present | 246 / 081 | Yes |
| 2.2 | 8 | 0x00 00 00 00 | 00001 | 11 | UAC_BarringInfo_Common (8,0x0010000'B) | 246 / 081 | No |
| 2.3 | 8 | 0x00 00 00 00 | 00001 | 11 | UAC_BarringInfo_PerPLMN (8,0x0000000'B) | 246 / 081 | Yes |
| 2.4 | 8 | 0x00 00 00 00 | 00001 | 11 | UAC_BarringInfo_Common (8,0x0000000'B) | 246 / 082 | No |
| 2.5 | 8 | 0x00 00 00 00 | 00001 | 11 | UAC_BarringInfo_Common2 (7,0x0000001'B, 8,0x0000000'B) | 244 / 081 | No |
| 2.6 | 8 | 0x00 00 00 00 | 10000 | 15 | UAC_BarringInfo_Common (8,0x0000001'B) | 246 / 081 | No |
| 2.7 | 8 | 0x00 00 00 00 | 10000 | 15 | UAC_BarringInfo_PerPLMN (8,0x0000000'B) | 246 / 081 | Yes |
| 2.8 | 8 | 0x00 00 00 00 | 10000 | 15 | UAC_BarringInfo_Common (8,0x0000000'B) | 246 / 082 | No |
| 2.9 | 8 | 0x00 00 00 00 | 10000 | 15 | UAC_BarringInfo_Common2 (7,0x0000001'B, 8,0x0000000'B) | 244 / 081 | No |

##### 5.4.8A.4.3	Procedure

Procedure/steps to be repeated for all sequences listed in table 5.4.8A-1:

| Step | Direction | Action | Comment | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | TT | SIB1 of Cell A is transmitted as defined in the initial conditions and in table 5.4.8A-1 | No barring info is provided to Cell A |  |  |
| 2 | UE | Power UE on | The UE performs a SIM initialisation | CR 1 | A.2/1 OR A.2/2 |
| 3 | UE > TT | Send RegistrationRequest |  |  |  |
| 4 | TT > UE | Send RegistrationAccept | The registration is successful |  |  |
| 5 | UE > TT | Set up MO Data call (PDU SessionEstablishment) | To set up the MO Data call the MMI or EMMI is used. The MO Data call is set up successfully |  |  |
| 6 | TT > UE | Send RRC RELEASE with suspendConfig in criticalExtensions (with the choice rrcRelease) | ran-NotificationAreaInfo in suspendConfig contains the cellList with cellIdentity of Cell A: - cellList { - plmn-Identity {mcc, mnc}, - -- see table 5.4.8A-1 for MCC/MNC - ran-AreaCells 000000001’B - } |  |  |
| 7 | TT | Deactivate Cell A and activate Cell B |  |  |  |
| 8 | TT | SIB1 of Cell B is transmitted as defined in table 5.4.8A-1 | Barring info is as in the table. See Annex A for the Methods UAC_BarringInfo_xxxxxx() in the table |  |  |
| 9 | UE > TT | Initiate RRC RESUME REQUEST procedure with resumeCause set to rna-Update. | See the column ‘RRCResumeRequest with resumeCause set to rna-Update successful?’ for the result | CR 2 CR 3 CR 4 CR 5 |  |

#### 5.4.8A.5	Acceptance criteria

For the scenarios in table 5.4.8A-1:

CR 1 is explicitly verified at step 2) by analysing the file READ commands for EFUAC-AIC and EFACC during the USIM-Terminal initialisation procedure.

CR 2, CR 3, CR 4, and CR 5 are verified

-	at step 9) by analysing if the UE shall make a successful or unsuccessful RRC Resumption for RNA Update in accordance with the result indicated in the table.

### 5.4.9	Unified Access Control – Access Identities 12, 13 and 14 indicated by USIM

#### 5.4.9.1	Definition and applicability

The purpose of Unified Access Control procedure is to perform access barring check for a 5GS access attempt associated with a given Access Category and one or more Access Identities upon request from upper layers or the RRC layer.

The 5G network shall be able to broadcast barring control information (i.e. a list of barring parameters associated with an Access Identity and an Access Category) in SIB1.

The EFACC in the USIM contains the configuration information pertaining to access identities 12, 13 and 14.

The UE shall be able to determine whether or not a particular new access attempt is allowed based on barring parameters that the UE receives from the broadcast barring control information and the configuration in the USIM.

When the NAS detects an access event, the NAS shall perform the mapping of the kind of request to one or more access identities and one access category and lower layers will perform access barring checks for that request based on the determined access identities and access category.

#### 5.4.9.2	Conformance requirement

CR 1	The ME shall read the access control value as part of the USIM-Terminal initialization procedure, and subsequently adopt this value.

Reference:

-	3GPP TS 31.102 [19], clause 5.1.1.2.

CR 2	Access Identities are configured at the UE as listed in 3GPP TS 22.261 [43] Table 6.22.2.2-1. Access Categories are defined by the combination of conditions related to UE and the type of access attempt as listed in 3GPP TS 22.261 [43] Table 6.22.2.3-1. One or more Access Identities and only one Access Category are selected and tested for an access attempt.

Reference:

-	3GPP TS 22.261 [43], clause 6.22.2.

CR 3	Access Identities 12, 13 and 14 are valid in HPLMN and visited PLMNs of home country only.

Reference:

-	3GPP TS 24.501 [42], clause 4.5.2.

CR 4	The UE shall be able to determine whether or not a particular new access attempt is allowed based on uac-BarringInfo broadcast in SIB1.  Access Control check shall be performed as per the information received in uac-BarringInfoSetList.

Reference:

-	3GPP TS 38.331 [44], clauses 5.3.14.

#### 5.4.9.3	Test purpose

The purpose of this test is to verify that:

1)	the ME reads the access control value from EFUAC-AIC and EFACC as part of the USIM-Terminal initialisation procedure, and subsequently adopts this value.

2)	the UE maps the kind of request to one or more access identities and one access category and lower layers performs access barring checks for that request based on the determined access identities and access category.

3)	the UE determines whether or not a particular access attempt is allowed based on uac-BarringInfo broadcast in SIB1 and if the RPLMN is the HPLMN, EHPLMN or visited PLMN of the home country.

#### 5.4.9.4	Method of test

##### 5.4.9.4.1	Initial conditions

The values of the 5G-NR UICC as defined in clause 4.5.9 of the present document are used with EFIMSI as defined in clause 4.6.5 and the following exceptions:

EFUAC_AIC and EFACC

No Access Identities configured in EFUAC_AIC and no Access Classes configured in EFACC as also shown in table 5.4.9-1.

EFUST (USIM Service Table)

Logically:

| Service n°126 |  | UAC Access Identities support | available |
| --- | --- | --- | --- |

Coding:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary: | xxxx xx1x | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxxx xxxx | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | … | B16 | B17 |  |  |
|  | xxxx xxxx | xxxx xxxx | xx11 xxxx | ... | xx10 11xx | 0000 0xxx |  |  |

The defined UICC/USIM data is installed on the UE.

The NG-SS is configured to transmit on the BCCH with the following parameters:

Cell A:

-	TAI (MCC/MNC/TAC):	MCC, MNC: see table 5.4.9-1, TAC: 000001.

-	CellIdentity :	000000001

For Table 5.4.9-1:

uac-BarringInfo in SIB1 should be set as in the table:

-	Refer to Annex A for the Methods UAC_BarringInfo_xxxxxx() in the table.

##### 5.4.9.4.2	Tables related to the test case

Table 5.4.9-1

| TC Seq# | Access Category | USIM |  |  | SIB1 |  | Result |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | EFUAC_AIC | EFACC (Byte 1 b8-b4) | Access Identity | uac-BarringInfo | PLMN-Identity (MCC/MNC) | Registration successful? | MO Data call successful? |
| 1.1 | 7 | 0x00 00 00 00 | 00010 | 12 | Not Present | 246 / 081 | Yes | Yes |
| 1.2 | 3 | 0x00 00 00 00 | 00010 | 12 | UAC_BarringInfo_Common (3,0x0001000'B) | 246 / 081 | No | NA |
| 1.3 | 3 | 0x00 00 00 00 | 00010 | 12 | UAC_BarringInfo_PerPLMN (3,0x0001000'B) | 246 / 081 | No | NA |
| 1.4 | 3 | 0x00 00 00 00 | 00010 | 12 | UAC_BarringInfo_Common (3,0x0000000'B) | 246 / 082 | Yes | Yes |
| 1.5 | 3 | 0x00 00 00 00 | 00010 | 12 | UAC_BarringInfo_Common (3,0x0001000'B) | 246 / 082 | No | NA |
| 1.6 | 3 | 0x00 00 00 00 | 00010 | 12 | UAC_BarringInfo_Common (3,0x0000000'B) | 244 / 081 | No | NA |
| 1.7 | 3 | 0x00 00 00 00 | 00010 | 12 | UAC_BarringInfo_Common (3,0x0000100'B) | 246 / 081 | Yes | Yes |
| 1.8 | 3 | 0x00 00 00 00 | 01110 | 12,13,14 | UAC_BarringInfo_Common (3,0x0000110'B) | 246 / 081 | Yes | Yes |
| 1.9 | 7 | 0x00 00 00 00 | 00010 | 12 | UAC_BarringInfo_Common (7,0x0001000'B) | 246 / 081 | Yes | No |
| 1.10 | 7 | 0x00 00 00 00 | 00010 | 12 | UAC_BarringInfo_Common (7,0x0000000'B) | 246 / 082 | Yes | Yes |
| 1.11 | 7 | 0x00 00 00 00 | 00010 | 12 | UAC_BarringInfo_Common (7,0x0000000'B) | 244 / 081 | Yes | No |
| 1.12 | 7 | 0x00 00 00 00 | 00010 | 12 | UAC_BarringInfo_Common2 (3,0x0000100'B, 7,0x0001000'B) | 246 / 081 | Yes | No |
| 1.13 | 3 | 0x00 00 00 00 | 00100 | 13 | UAC_BarringInfo_Common (3,0x0000100'B) | 246 / 081 | No | NA |
| 1.14 | 3 | 0x00 00 00 00 | 00100 | 13 | UAC_BarringInfo_PerPLMN (3,0x0000100'B) | 246 / 081 | No | NA |
| 1.15 | 3 | 0x00 00 00 00 | 00100 | 13 | UAC_BarringInfo_Common (3,0x0000000'B) | 246 / 082 | Yes | Yes |
| 1.16 | 3 | 0x00 00 00 00 | 00100 | 13 | UAC_BarringInfo_Common (3,0x0000100'B) | 246 / 082 | No | NA |
| 1.17 | 3 | 0x00 00 00 00 | 00100 | 13 | UAC_BarringInfo_Common (3,0x0000010'B) | 246 / 082 | Yes | Yes |
| 1.18 | 7 | 0x00 00 00 00 | 00100 | 13 | UAC_BarringInfo_Common (7,0x0000100'B) | 246 / 081 | Yes | No |
| 1.19 | 7 | 0x00 00 00 00 | 00100 | 13 | UAC_BarringInfo_Common (7,0x0000000'B) | 244 / 081 | Yes | No |
| 1.20 | 7 | 0x00 00 00 00 | 00100 | 13 | UAC_BarringInfo_Common2 (3,0x0000010'B, 7,0x0000100'B) | 246 / 081 | Yes | No |
| 1.21 | 3 | 0x00 00 00 00 | 01000 | 14 | UAC_BarringInfo_Common (3,0x0000010'B) | 246 / 081 | No | NA |
| 1.22 | 3 | 0x00 00 00 00 | 01000 | 14 | UAC_BarringInfo_PerPLMN (3,0x0000010'B) | 246 / 081 | No | NA |
| 1.23 | 3 | 0x00 00 00 00 | 01000 | 14 | UAC_BarringInfo_Common (3,0x0000000'B) | 246 / 082 | Yes | Yes |
| 1.24 | 3 | 0x00 00 00 00 | 01000 | 14 | UAC_BarringInfo_Common (3,0x0000010'B) | 246 / 082 | No | NA |
| 1.25 | 3 | 0x00 00 00 00 | 01000 | 14 | UAC_BarringInfo_Common (3,0x0001100'B) | 246 / 082 | Yes | Yes |
| 1.26 | 7 | 0x00 00 00 00 | 01000 | 14 | UAC_BarringInfo_Common (7,0x0000010'B) | 246 / 081 | Yes | No |
| 1.27 | 7 | 0x00 00 00 00 | 01000 | 14 | UAC_BarringInfo_Common (7,0x0000000'B) | 244 / 081 | Yes | No |
| 1.28 | 7 | 0x00 00 00 00 | 01000 | 14 | UAC_BarringInfo_Common2 (3,0x0000100'B, 7,0x0000010'B) | 246 / 081 | Yes | No |

##### 5.4.9.4.3	Procedure

Procedure/steps to be repeated for all sequences listed in table 5.4.9-1:

| Step | Direction | Action | Comment | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | TT | SIB1 of Cell A is transmitted as defined in the initial conditions for tests from table 5.4.9-1 | Barring info is set as in table 5.4.9-1 See Annex A for the Methods UAC_BarringInfo_xxxxxx() in the table |  |  |
| 2 | UE | Power UE on | The UE performs a SIM initialisation. (includes reading of EFUAC-AIC and EFACC) | CR 1 | A.2/1 OR A.2/2 |

| If cell access is not allowed: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 3 | UE > TT | REGISTRATION REQUEST is not sent | See column ‘Registration successful?’ for the result | CR 2 CR 3 CR 4 |  |
| 4 | Tester | End test sequence |  |  |  |

| If cell access is allowed: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 3 | UE > TT | Send REGISTRATION REQUEST | See column ‘Registration successful?’ for the result | CR 2 CR 3 CR 4 |  |
| 4 | TT > UE | Send REGISTRATION ACCEPT | For simplicity other signalling is not shown |  |  |

| If MO Data call is not allowed: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 5 | UE | Attempt to set up MO Data call | To set up the MO Data call the MMI or EMMI is used | CR 2 CR 3 CR 4 |  |
| 6 | Tester | End test sequence |  |  |  |

| If MO Data call is allowed: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 5 | UE > TT | Set up MO Data call (PDU SessionEstablishment) | To set up the MO Data call the MMI or EMMI is used | CR 2 CR 3 CR 4 |  |
| 6 | Tester | End test sequence |  |  |  |

#### 5.4.9.5	Acceptance criteria

For the scenarios in table 5.4.9-1:

CR 1 is explicitly verified at step 2) by analysing the file READ commands for EFUAC-AIC and EFACC during the USIM-ME initialisation procedure.

CR 2, CR 3, and CR 4 are verified

- at step 3) by analysing if the UE shall make a successful or unsuccessful registration to the network in accordance with the result indicated in the table and

- at step 5) by analysing if the UE shall make a successful or unsuccessful MO Data call in accordance with the result indicated in the table if the step is applicable.

### 5.4.9A	Unified Access Control – Access Identities 12, 13 and 14 indicated by USIM, Access Category 8

#### 5.4.9A.1	Definition and applicability

The purpose of Unified Access Control procedure is to perform access barring check for a 5GS access attempt associated with a given Access Category and one or more Access Identities upon request from upper layers or the RRC layer.

The 5G network shall be able to broadcast barring control information (i.e. a list of barring parameters associated with an Access Identity and an Access Category) in SIB1.

The EFACC in the USIM contains the configuration information pertaining to access identities 12, 13 and 14.

The UE shall be able to determine whether or not a particular new access attempt is allowed based on barring parameters that the UE receives from the broadcast barring control information and the configuration in the USIM.

When the NAS detects an access event, the NAS shall perform the mapping of the kind of request to one or more access identities and one access category and lower layers will perform access barring checks for that request based on the determined access identities and access category.

If RRC state is RRC_INACTIVE and the resumption of the RRC connection is triggered due to an RNA Update RRC layer shall select Access Category as 8 and perform unified access control procedure in case there is no ongoing emergency service.

#### 5.4.9A.2	Conformance requirement

CR 1	The ME shall read the access control value as part of the USIM-Terminal initialization procedure, and subsequently adopt this value.

Reference:

-	3GPP TS 31.102 [19], clause 5.1.1.2.

CR 2	Access Identities are configured at the UE as listed in 3GPP TS 22.261 [43] Table 6.22.2.2-1. Access Categories are defined by the combination of conditions related to UE and the type of access attempt as listed in 3GPP TS 22.261 [43] Table 6.22.2.3-1. One or more Access Identities and only one Access Category are selected and tested for an access attempt.

Reference:

-	3GPP TS 22.261 [43], clause 6.22.2.

CR 3	Access Identities 12, 13 and 14 are valid in HPLMN and visited PLMNs of home country only.

Reference:

-	3GPP TS 24.501 [42], clause 4.5.2.

CR 4	The UE shall be able to determine whether or not a particular new access attempt is allowed based on uac-BarringInfo broadcast in SIB1.  Access Control check shall be performed as per the information received in uac-BarringInfoSetList.

Reference:

-	3GPP TS 38.331 [44], clauses 5.3.14.

CR 5	If the resumption of the RRC connection is triggered due to an RNA update and there is no ongoing emergency service RRC shall select '8' as the Access Category and perform the unified access control procedure.

Reference:

-	3GPP TS 38.331 [44], clauses 5.3.13.2.

#### 5.4.9A.3	Test purpose

The purpose of this test is to verify that:

1)	the ME reads the access control value from EFUAC-AIC and EFACC as part of the USIM-Terminal initialisation procedure, and subsequently adopts this value.

2)	the UE maps the kind of request to one or more access identities and one access category and lower layers performs access barring checks for that request based on the determined access identities and access category.

3)	the UE determines whether or not a particular access attempt is allowed based on uac-BarringInfo broadcast in SIB1 and if the RPLMN is the HPLMN, EHPLMN or visited PLMN of the home country.

4)	the UE performs unified access control procedure if RNA Update procedure is triggered.

#### 5.4.9A.4	Method of test

##### 5.4.9A.4.1	Initial conditions

The values of the 5G-NR UICC as defined in clause 4.5.9 of the present document are used with EFIMSI as defined in clause 4.6.5 and the following exceptions:

EFUAC_AIC and EFACC

No Access Identities configured in EFUAC_AIC and no Access Classes configured in EFACC as also shown in table 5.4.9-1.

EFUST (USIM Service Table)

Logically:

| Service n°126 |  | UAC Access Identities support | available |
| --- | --- | --- | --- |

Coding:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary: | xxxx xx1x | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxxx xxxx | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | … | B16 | B17 |  |  |
|  | xxxx xxxx | xxxx xxxx | xx11 xxxx | ... | xx10 11xx | 0000 0xxx |  |  |

The defined UICC/USIM data is installed on the UE.

The TT (NG-SS) is configured to transmit on the BCCH with the following parameters:

Cell A:

-	TAI (MCC/MNC/TAC):	MCC, MNC: see table 5.4.9A-1, TAC: 000001.

-	CellIdentity:	000000001

For Table 5.4.9A-1:

No uac-BarringInfo in SIB1.

Cell B:

-	TAI (MCC/MNC/TAC):	MCC, MNC: see table 5.4.9A-1, TAC: 000001.

-	CellIdentity:	000000002

For Table 5.4.9A-1:

uac-BarringInfo in SIB1 should be set as in the table:

-	Refer to Annex A for the Methods UAC_BarringInfo_xxxxxx() in the table.

##### 5.4.9A.4.2	Tables related to the test case

Table 5.4.9A-1

| TC Seq# | Access Category | USIM |  |  | SIB1 |  | Result |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | EFUAC_AIC | EFACC (Byte 1 b8-b4) | Access Identity | uac-BarringInfo | PLMN-Identity (MCC/MNC) | RRCResumeRequest with resumeCause set to rna-Update successful? |
| 2.1 | 8 | 0x00 00 00 00 | 00010 | 12 | Not Present | 246 / 081 | Yes |
| 2.2 | 8 | 0x00 00 00 00 | 00010 | 12 | UAC_BarringInfo_Common( 8,0x0001000'B) | 246 / 081 | No |
| 2.3 | 8 | 0x00 00 00 00 | 00010 | 12 | UAC_BarringInfo_PerPLMN( 8,0x0000000'B) | 246 / 081 | Yes |
| 2.4 | 8 | 0x00 00 00 00 | 00010 | 12 | UAC_BarringInfo_Common( 8,0x0010000'B) | 246 / 082 | Yes |
| 2.5 | 8 | 0x00 00 00 00 | 00010 | 12 | UAC_BarringInfo_Common2 ( 7,0x0010000'B, 8,0x0000000'B) | 244 / 081 | No |
| 2.6 | 8 | 0x00 00 00 00 | 00100 | 13 | UAC_BarringInfo_Common( 8,0x0000100'B) | 246 / 081 | No |
| 2.7 | 8 | 0x00 00 00 00 | 00100 | 13 | UAC_BarringInfo_PerPLMN( 8,0x0000000'B) | 246 / 081 | Yes |
| 2.8 | 8 | 0x00 00 00 00 | 00100 | 13 | UAC_BarringInfo_Common( 8,0x0000100'B) | 246 / 082 | No |
| 2.9 | 8 | 0x00 00 00 00 | 00100 | 13 | UAC_BarringInfo_Common2 ( 7,0x0001000'B, 8,0x0000000'B) | 244 / 081 | No |
| 2.10 | 8 | 0x00 00 00 00 | 01000 | 14 | UAC_BarringInfo_Common( 8,0x0000010'B) | 246 / 081 | No |
| 2.11 | 8 | 0x00 00 00 00 | 01000 | 14 | UAC_BarringInfo_PerPLMN( 8,0x0000000'B) | 246 / 081 | Yes |
| 2.12 | 8 | 0x00 00 00 00 | 01000 | 14 | UAC_BarringInfo_Common( 8,0x0000010'B) | 246 / 082 | No |
| 2.13 | 8 | 0x00 00 00 00 | 01000 | 14 | UAC_BarringInfo_Common2 ( 7,0x0000100'B, 8,0x0000000'B) | 244 / 081 | No |

##### 5.4.9A.4.3	Procedure

Procedure/steps to be repeated for all sequences listed in table 5.4.9A-1:

| Step | Direction | Action | Comment | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | TT | SIB1 of Cell A is transmitted as defined in the initial conditions and in table 5.4.9A-1 | No barring info is provided to Cell A |  |  |
| 2 | UE | Power UE on | The UE performs a SIM initialisation | CR 1 | A.2/1 OR A.2/2 |
| 3 | UE > TT | Send REGISTRATION REQUEST |  |  |  |
| 4 | TT > UE | Send REGISTRATION ACCEPT | The registration is successful |  |  |
| 5 | UE > TT | Set up MO Data call (PDU SessionEstablishment) | To set up the MO Data call the MMI or EMMI is used. The MO Data call is set up successfully |  |  |
| 6 | TT > UE | Send RRC RELEASE with suspendConfig in criticalExtensions (with the choice rrcRelease) | ran-NotificationAreaInfo in suspendConfig contains the cellList with cellIdentity of Cell A: - cellList { - plmn-Identity {mcc, mnc}, - -- see table 5.4.9A-1 for MCC/MNC - ran-AreaCells 000000001’B } |  |  |
| 7 | TT | Deactivate Cell A and activate Cell B |  |  |  |
| 8 | TT | SIB1 of Cell B is transmitted as defined in table 5.4.9A-1 | Barring info is as in the table. See Annex A for the Methods UAC_BarringInfo_xxxxxx() in the table |  |  |
| 9 | UE > TT | Initiate RRC RESUME REQUEST procedure with resumeCause set to rna-Update. | See the column ‘RRCResumeRequest with resumeCause set to rna-Update successful?’ for the result | CR 2 CR 3 CR 4 CR 5 |  |

#### 5.4.9A.5	Acceptance criteria

For the scenarios in table 5.4.9A-1:

CR 1 is explicitly verified at step 2) by analysing the file READ commands for EFUAC-AIC and EFACC during the USIM-Terminal initialisation procedure.

CR 2, CR 3, CR 4, and CR 5 are verified

-	at step 9) by analysing if the UE shall make a successful or unsuccessful RRC Resumption for RNA Update in accordance with the result indicated in the table.

### 5.4.10	Unified Access Control – Operator-Defined Access Category

#### 5.4.10.1	Definition and applicability

The purpose of Unified Access Control procedure is to perform access barring check for a 5GS access attempt associated with a given Access Category and one or more Access Identities upon request from upper layers or the RRC layer.

The 5G network shall be able to broadcast barring control information a list of barring parameters associated with an Access Identity and an Access Category in SIB1. Access category numbers in the 32-63 range are used for operator-defined Access Categories.

The EFUAC_AIC in the USIM contains the configuration information pertaining to access identities allocated for high priority services that can be used by the subscriber.

The UE shall be able to determine whether or not a particular new access attempt is allowed based on barring parameters that the UE receives from the broadcast barring control information and the configuration in the USIM.

When the NAS detects an access event, the NAS shall perform the mapping of the kind of request to one or more access identities and one access category and lower layers will perform access barring checks for that request based on the determined access identities and access category.

Operator-defined access category definitions can be signaled to the UE using NAS signaling. Each operator-defined access category definition consists of the parameters a precedence value, an operator-defined access category number, one or more access category criteria type and optionally, a standardized access category.

When the UE needs to initiate an access attempt that triggers access barring check, the UE shall determine one or more access identities from the set of standardized access identities, and one access category from the set of standardized access categories and operator-defined access categories, to be associated with that access attempt.

In order to determine the access category applicable for the access attempt, the NAS shall check the Access category mapping rules, and use the access category of the lowest rule number for which there is a match for barring check. In the case of operator-defined access categories, the UE shall select the access category from the operator-defined access category definition with the lowest precedence value.

#### 5.4.10.2	Conformance requirement

CR 1	The ME shall read the access control value as part of the USIM initialization procedure, and subsequently adopt this value.

Reference:

-	3GPP TS 31.102 [19], clause 5.1.1.2.

CR 2	Access Identities are configured at the UE as listed in Table 6.22.2.2-1. Access Categories are defined by the combination of conditions related to UE and the type of access attempt as listed in Table 6.22.2.3-1.

Reference:

-	3GPP TS 22.261 [43], clause 6.22.2

CR 3	Operator may provide one or more PLMN-specific Operator-defined access category definitions to the UE using NAS signalling (Operator-defined access category definitions information element), and the UE handles the Operator-defined access category definitions stored for the Registered PLMN, as specified in TS 24.501.

References:

-	3GPP TS 23.501 [49], clause 5.2.5

-	3GPP TS 24.501 [25] clause 9.11.3.38

CR 4	When the UE needs to initiate an access attempt in one of the events listed in clause 4.5.1, the UE shall determine one or more access identities from the set of standardized access identities, and one access category from the set of standardized access categories and operator-defined access categories, to be associated with that access attempt.

Reference:

-	3GPP TS 24.501 [25], clause 4.5.2

CR 5	An access attempt matches the criteria of an operator-defined access category definition, if the access attempt matches all access category criteria types included in the criteria with any of the associated access criteria type values.

Reference:

-	3GPP TS 24.501 [25], clause 4.5.3

CR 6	In order to determine the access category applicable for the access attempt, the NAS shall check the rules in table 4.5.2.2, and use the access category for which there is a match for barring check. If the access attempt matches more than one rule, the access category of the lowest rule number shall be selected. If the access attempt matches more than one operator-defined access category definition, the UE shall select the access category from the operator-defined access category definition with the lowest precedence value (see clause 4.5.3).

Reference:

-	3GPP TS 24.501 [25], clause 4.5.2

CR 7	If the UE receives Operator-defined access category definitions IE in the REGISTRATION ACCEPT message and the Operator-defined access category definitions IE contains one or more operator-defined access category definitions, the UE shall delete any operator-defined access category definitions stored for the RPLMN and shall store the received operator-defined access category definitions for the RPLMN. If the UE receives the Operator-defined access category definitions IE in the REGISTRATION ACCEPT message and the Operator-defined access category definitions IE contains no operator-defined access category definitions, the UE shall delete any operator-defined access category definitions stored for the RPLMN. If the REGISTRATION ACCEPT message does not contain the Operator-defined access category definitions IE, the UE shall not delete the operator-defined access category definitions stored for the RPLMN.

Reference:

-	3GPP TS 24.501 [25], clause 5.5.1.2.4

CR 8	The UE shall be able to determine whether or not a particular new access attempt is allowed based on uac-BarringInfo broadcast in SIB1.  Access Control check shall be performed as per the information received in uac-BarringInfoSetList.

Reference:

-	3GPP TS 38.331 [37], clauses 5.3.14

#### 5.4.10.3	Test purpose

The purpose of this test is to verify that:

1)	the ME reads the access control value from EFUAC_AIC and EFACC as part of the USIM initialisation procedure, and subsequently adopts this value.

2)	if the UE uses operator-defined access categories received within Operator-defined access category definitions information element in the REGISTRATION ACCEPT.

3)	the UE checks the access category applicable for the access attempt, as per the Access category mapping rules and chooses the matching lowest rule.

4)	the UE checks operator-defined access category parameters precedence value, operator-defined access category number and access category criteria type associated with the Operator-defined Access Category when determining the Access Category for the access attempt.

5)	the UE stops using the operator-defined access category definitions stored for the previously selected PLMN.

6)	the UE maps the kind of request to one or more access identities and one access category and lower layers performs access barring checks for that request based on the determined access identities and access category.

7)	the UE determines whether or not a particular access attempt is allowed based on uac-BarringInfo broadcast in SIB1 and if the RPLMN is the HPLMN, EHPLMN or visited PLMN of the home country.

#### 5.4.10.4	Method of test

##### 5.4.10.4.1	Initial conditions

The values of the 5G-NR UICC as defined in clause 4.5.9 of the present document are used with EFIMSI as defined in clause 4.6.5 and the following exceptions:

EFUAC_AIC and EFACC

No Access Identities configured in EFUAC_AIC and no Access Classes configured in EFACC as also shown in table 5.4.10-1.

EFUST (USIM Service Table)

Logically:

| Service n°126 |  | UAC Access Identities support | available |
| --- | --- | --- | --- |

Coding:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary: | xxxx xx1x | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxxx xxxx | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | … | B16 | B17 |  |  |
|  | xxxx xxxx | xxxx xxxx | xx11 xxxx | ... | xx10 11xx | 0000 0xxx |  |  |

ME shall be configured with following URSP rules:

Rule Precedence = 1

Traffic Descriptor:

DNN=TestGp.rs1

Route Selection Descriptor:

Precedence = 1

Network Slice Selection, S-NSSAI:	′01 01 01 01′ (ST: MBB, SD: 010101)

SSC Mode Selection:					SSC Mode 1

Access Type preference:				3GPP access

Rule Precedence = 2

Traffic Descriptor:

DNN = TestGp.rs2

Route Selection Descriptor:

Precedence = 1

Network Slice Selection, S-NSSAI:	′01 01 01 02′ (ST: MBB, SD: 010102)

SSC Mode Selection:					SSC Mode 1

Rule Precedence = 3

Traffic Descriptor:

DNN = TestGp.rs3

Route Selection Descriptor:

Precedence = 1

Network Slice Selection, S-NSSAI:	′01 01 01 02′ (ST: MBB, SD: 010102)

SSC Mode Selection:					SSC Mode 1

The TT (NG-SS) is configured to transmit on the BCCH with the following parameters:

Cell A:

-	MCC, MNC:	246/081

-	TAC: 			000001

-	CellIdentity:	000000001

For Table 5.4.10-1:

After turning on the cell for the 2nd time, uac-BarringInfo in SIB1 should be set as in the table:

-	Refer to Annex A for the Methods UAC_BarringInfo_xxxxxx() in the table.

REGISTRATION ACCEPT IEIs.

-	Operator-Defined Access Category Definitions IEI is configured as defined in Table 5.4.10-1

-	Refer to Annex A tables A4-A5 for Methods ODAC_definitions1() and ODAC_definitions2().

-	For HPLMN Cells: Allowed S-NSSAI IEI is configured to include S-NSSAIs ′01 01 01 01′, ′01 01 01 02′ and ′01 01 01 03′.

For VPLMN Cells:

-	Allowed S-NSSAI IEI is configured to include S-NSSAIs ('01 01 01 01', '01 01 01 01'), ('01 01 01 02', '01 01 01 02'), and ('01 01 01 03', '01 01 01 03').

##### 5.4.10.4.2	Tables related to the test case

Table 5.4.10-1

| TC Seq# | Access Category for Call 1&2 | USIM |  | SIB1 of Cell A |  | REGISTRATION ACCEPT | Result |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | EFUAC_AIC | EFACC (Byte 1 b8-b4) | uac-BarringInfo | PLMN-Identity (MCC/MNC) | (Operator-defined access category definitions IE) | MO Data call 1 successful? | MO Data call 2 successful? |
| 1.1 | 32,7 | 0x00 00 00 00 | 00000 | Not Present | 246 / 081 | ODAC_definitions1(0) | Yes | Yes |
| 1.2 | 32,7 | 0x00 00 00 00 | 00000 | UAC_BarringInfo_Common (32,0x0000000'B) | 246 / 081 | ODAC_definitions1(0) | No | Yes |
| 1.3 | 50,7 | 0x00 00 00 00 | 00000 | UAC_BarringInfo_Common (50,0x0100000'B) | 246 / 081 | ODAC_definitions1(18) | No | Yes |
| 1.4 | 63,7 | 0x01 00 00 00 | 00000 | UAC_BarringInfo_Common (7,0x1000000'B) | 246 / 082 | ODAC_definitions1(31) | Yes | No |
| 1.5 | 63,7 | 0x02 00 00 00 | 00000 | UAC_BarringInfo_Common (63,0x0100000'B) | 246 / 081 | ODAC_definitions1(31) | No | Yes |
| 1.6 | 48,7 | 0x01 00 00 00 | 00000 | UAC_BarringInfo_Common2 (7,0x1000000'B, 48,0x1000000'B) | 246 / 082 | ODAC_definitions1(16) | No | No |
| 1.7 | 48,7 | 0x02 00 00 00 | 00000 | UAC_BarringInfo_Common (48,0x1000000'B) | 246 / 082 | ODAC_definitions1(16) | Yes | Yes |
| 1.8 | 50,7 | 0x02 00 00 00 | 00000 | UAC_BarringInfo_PerPLMN (50,0x1100000'B) | 244 / 081 | ODAC_definitions1(18) | No | Yes |
| 1.9 | 50,7 | 0x00 00 00 00 | 00001 | UAC_BarringInfo_Common (50,0x0001000'B) | 244 / 081 | ODAC_definitions1(18) | No | Yes |
| 1.10 | 33,7 | 0x02 00 00 00 | 00000 | UAC_BarringInfo_Common2 (33,0x0100000'B, 7,0x1000000'B) | 246 / 082 | ODAC_definitions1(1) | No | Yes |
| 1.11 | 34,7 | 0x01 00 00 00 | 00000 | UAC_BarringInfo_Common2 (7,0x1000000'B, 34,0x0100000'B) | 246 / 082 | ODAC_definitions1(2) | Yes | No |
| 1.12 | 7,34 | 0x01 00 00 00 | 00000 | UAC_BarringInfo_Common2 (32,0x1000000'B, 34,0x0100000'B) | 246 / 081 | ODAC_definitions2(2,0) | Yes | Yes |
| 1.13 | 7,32 | 0x00 00 00 00 | 01000 | UAC_BarringInfo_Common2 (32,0x0000000'B, 34,0x0000010'B) | 246 / 082 | ODAC_definitions2(0,2) | Yes | Yes |
| 1.14 | 7,48 | 0x00 00 00 00 | 00100 | UAC_BarringInfo_Common2 (48,0x0100000'B, 7,0x0000100'B) | 246 / 082 | ODAC_definitions2(16,31) | No | Yes |
| 1.15 | 7,32 | 0x01 00 00 00 | 00000 | UAC_BarringInfo_Common2 (32,0x1000000'B, 34,0x0100000'B) | 246 / 081 | ODAC_definitions2(2,0) | Yes | No |
| 1.16 | 7,34 | 0x00 00 00 00 | 01000 | UAC_BarringInfo_Common2 (32,0x0000000'B, 34,0x0000010'B) | 246 / 082 | ODAC_definitions2(0,2) | Yes | No |
| 1.17 | 7,63 | 0x00 00 00 00 | 00100 | UAC_BarringInfo_Common2 (48,0x0100000'B, 7,0x0000100'B) | 246 / 082 | ODAC_definitions2(16,31) | No | Yes |

##### 5.4.10.4.3	Procedure

Procedure/steps to be repeated for all sequences listed in table 5.4.10-1:

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | TT | SIB1 of Cell A is transmitted as defined in the initial conditions for tests from table 5.4.10-1 | Barring info is set as in table 5.4.10-1 See Annex A for the Methods UAC_BarringInfo_xxxxxx() in the table |  |  |
| 2 | USER | Power UE on | The UE performs a SIM initialisation. (includes reading of EFUAC-AIC and EFACC) | CR 1 | A.2/1 OR A.2/2 |
| 3 | UE > TT | Send REGISTRATION REQUEST |  |  |  |
| 4 | TT > UE | Send REGISTRATION ACCEPT | REGISTRATION ACCEPT with the Operator-Defined Access Category Definitions IEI as defined in the table 5.4.10-1. For simplicity other signalling is not shown |  |  |

| If 1st MO Data call is not allowed: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 5 | UE > TT | Attempt to set up 1st MO Data call | Attempt to set up the MO Data call the MMI or EMMI is used | CR 2 to CR 8 |  |

| If 1st MO Data call is allowed: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 5 | UE > TT | Set up MO Data call (PDU SessionEstablishment) | To set up the MO Data call the MMI or EMMI is used | CR 2 to CR 8 |  |

| For sequences 1.1 – 1.14: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| If 2nd MO Data call is not allowed: |  |  |  |  |  |
| 6 | UE > TT | Attempt to set up 2nd MO Data call | Attempt to set up the MO Data call to DNN TestGp.rs2/S-NSSAI '01 01 01 02' using the MMI or EMMI. | CR 2 to CR 8 |  |
| 7 | USER | End test sequence |  |  |  |

| For sequences 1.1 – 1.14: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| If 2nd MO Data call is allowed: |  |  |  |  |  |
| 6 | UE > TT | Set up MO Data call (PDU SessionEstablishment) | To set up the MO Data call to DNN TestGp.rs2/S-NSSAI '01 01 01 02' using the MMI or EMMI. | CR 2 to CR 8 |  |
| 7 | USER | End test sequence |  |  |  |

| For sequences 1.15 – 1.17: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| If 2nd MO Data call is not allowed: |  |  |  |  |  |
| 6 | UE > TT | Attempt to set up 2nd MO Data call | Attempt to set up the MO Data call to DNN TestGp.rs3/S-NSSAI '01 01 01 02' using the MMI or EMMI. | CR 2 to CR 8 |  |
| 7 | USER | End test sequence |  |  |  |

| For sequences 1.15 – 1.17: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| If 2nd MO Data call is allowed: |  |  |  |  |  |
| 6 | UE > TT | Set up MO Data call (PDU SessionEstablishment) | To set up the MO Data call to DNN TestGp.rs3/S-NSSAI '01 01 01 02' using the MMI or EMMI. | CR 2 to CR 8 |  |
| 7 | USER | End test sequence |  |  |  |

#### 5.4.10.5	Acceptance criteria

For the scenarios in table 5.4.10-1:

CR 1 is explicitly verified at step 2) by analysing the file READ commands for EFUAC-AIC and EFACC during the USIM-Terminal initialisation procedure.

CR 2, through CR 8 are verified

- at steps 5) by analysing if the UE shall make a successful or unsuccessful 1st MO Data call in accordance with the result indicated in the table if the step is applicable and

- at steps 6) and 7) by analysing if the UE shall make a successful or unsuccessful 2nd MO Data call in accordance with the result indicated in the table if the step is applicable.

### 5.4.11	Unified Access Control – Operator-Defined Access Categories, no change in SUPI

#### 5.4.11.1	Definition and applicability

The purpose of Unified Access Control procedure is to perform access barring check for a 5GS access attempt associated with a given Access Category and one or more Access Identities upon request from upper layers or the RRC layer.

The 5G network shall be able to broadcast barring control information a list of barring parameters associated with an Access Identity and an Access Category in SIB1. Access category numbers in the 32-63 range are used for operator-defined Access Categories.

The EFUAC_AIC in the USIM contains the configuration information pertaining to access identities allocated for high priority services that can be used by the subscriber.

The UE shall be able to determine whether or not a particular new access attempt is allowed based on barring parameters that the UE receives from the broadcast barring control information and the configuration in the USIM.

Operator-defined access category definitions can be signalled to the UE using NAS signalling. Each operator-defined access category definition consists of the parameters: a precedence value, an operator-defined access category number, one or more access category criteria type and optionally, a standardized access category.

Operator-defined access category definitions are stored together with a PLMN identity of the PLMN that provided them, and is valid in that PLMN or equivalent PLMN. The operator-defined access category definitions can only be used if the SUPI from the USIM matches the SUPI stored in the non-volatile memory of the ME, else the UE shall delete the operator-defined access category definitions.

In order to determine the access category applicable for the access attempt, the NAS shall check the Access category mapping rules, and use the access category of the lowest rule number for which there is a match for barring check. In the case of operator-defined access categories, the UE shall select the access category from the operator-defined access category definition with the lowest precedence value.

#### 5.4.11.2	Conformance requirement

CR 1	The ME shall read the access control value as part of the USIM initialization procedure, and subsequently adopt this value.

Reference:

-	3GPP TS 31.102 [19], clause 5.1.1.2.

CR 2	Access Identities are configured at the UE as listed in 3GPP TS 22.261 [43] Table 6.22.2.2-1. Access Categories are defined by the combination of conditions related to UE and the type of access attempt as listed in 3GPP TS 22.261 [36] Table 6.22.2.3-1.

Reference:

-	3GPP TS 22.261 [36], clause 6.22.2

CR 3	Operator may provide one or more PLMN-specific Operator-defined access category definitions to the UE using NAS signalling (Operator-defined access category definitions information element), and the UE handles the Operator-defined access category definitions stored for the Registered PLMN, as specified in 3GPP TS 24.501.

References:

-	3GPP TS 23.501 [39], clause 5.2.5

-	3GPP TS 24.501 [25] clause 9.11.3.38

CR 4	Operator-defined access category definitions are stored together with a PLMN identity of the PLMN that provided them, and is valid in that PLMN or equivalent PLMN. The operator-defined access category definitions can only be used if the SUPI from the USIM matches the SUPI stored in the non-volatile memory of the ME, else the UE shall delete the operator-defined access category definitions.

Reference:

-	3GPP TS 24.501 [25], Annex C

CR 5	In order to determine the access category applicable for the access attempt, the NAS shall check the rules in 3GPP TS 24.501 [42] Table 4.5.2.2, and use the access category for which there is a match for barring check. If the access attempt matches more than one rule, the access category of the lowest rule number shall be selected. If the access attempt matches more than one operator-defined access category definition, the UE shall select the access category from the operator-defined access category definition with the lowest precedence value (see 3GPP TS 24.501 [42] clause 4.5.3).

Reference:

-	3GPP TS 24.501 [25], clause 4.5.2

CR 6	If the UE receives Operator-defined access category definitions IE in the REGISTRATION ACCEPT message and the Operator-defined access category definitions IE contains one or more operator-defined access category definitions, the UE shall delete any operator-defined access category definitions stored for the RPLMN and shall store the received operator-defined access category definitions for the RPLMN. If the UE receives the Operator-defined access category definitions IE in the REGISTRATION ACCEPT message and the Operator-defined access category definitions IE contains no operator-defined access category definitions, the UE shall delete any operator-defined access category definitions stored for the RPLMN. If the REGISTRATION ACCEPT message does not contain the Operator-defined access category definitions IE, the UE shall not delete the operator-defined access category definitions stored for the RPLMN.

Reference:

-	3GPP TS 24.501 [25], clause 5.5.1.2.4

CR 7	The UE shall be able to determine whether or not a particular new access attempt is allowed based on uac-BarringInfo broadcast in SIB1.  Access Control check shall be performed as per the information received in uac-BarringInfoSetList.

Reference:

-	3GPP TS 38.331 [37], clauses 5.3.14

#### 5.4.11.3	Test purpose

The purpose of this test is to verify that:

1)	the ME reads the access control value from EFUAC_AIC and EFACC as part of the USIM-Terminal initialisation procedure, and subsequently adopts this value.

2)	if UE uses operator-defined access categories received within Operator-defined access category definitions information element in the REGISTRATION ACCEPT.

3)	the UE checks the access category applicable for the access attempt, as per the Access category mapping rules and chooses the matching lowest rule.

4)	the UE checks operator-defined access category parameters precedence value, operator-defined access category number and access category criteria type associated with the Operator-defined Access Category when determining the Access Category for the access attempt.

5)	the Operator-defined access category definitions are stored together with a PLMN identity of the PLMN that provided them, and is valid in that PLMN or equivalent PLMN.

6)	the Operator-defined access category definitions can only be used if the SUPI from the USIM matches the SUPI stored in the non-volatile memory of the ME, else the UE shall delete the operator-defined access category definitions.

7)	the UE maps the kind of request to one or more access identities and one access category and lower layers performs access barring checks for that request based on the determined access identities and access category.

8)	the UE determines whether or not a particular access attempt is allowed based on uac-BarringInfo broadcast in SIB1 and if the RPLMN is the HPLMN, EHPLMN or visited PLMN of the home country.

#### 5.4.11.4	Method of test

##### 5.4.11.4.1	Initial conditions

The values of the 5G-NR UICC as defined in clause 4.5.9 of the present document are used with EFIMSI as defined in clause 4.6.5 and the following exceptions:

EFUAC_AIC and EFACC

Access Identity is configured in EFUAC_AIC and no Access Classes are configured in EFACC as given in Table 5.4.11-1.

EFUST (USIM Service Table)

Logically:

| Service n°126 |  | UAC Access Identities support | available |
| --- | --- | --- | --- |

Coding:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary: | xxxx xx1x | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxxx xxxx | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | … | B16 | B17 |  |  |
|  | xxxx xxxx | xxxx xxxx | xx11 xxxx | ... | xx10 11xx | 0000 0xxx |  |  |

ME shall be configured with following URSP rules:

Rule Precedence =1

Traffic Descriptor:

DNN=TestGp.rs1

Route Selection Descriptor:

Precedence=1

Network Slice Selection, S-NSSAI: '01 01 01 01' (ST: MBB, SD: 010101)

SSC Mode Selection: SSC Mode 1

Access Type preference: 3GPP access

Rule Precedence = 2

Traffic Descriptor:

DNN=TestGp.rs2

Route Selection Descriptor:

Precedence =1

Network Slice Selection, S-NSSAI: '01 01 01 02' (ST: MBB, SD: 010102)

SSC Mode Selection: SSC Mode 1

The TT (NG-SS) is configured to transmit on the BCCH with the following parameters:

Cell A:

-	MCC, MNC:	246/081

-	TAC:			000001

-	CellIdentity:	000000001

For Table 5.4.11-1:

uac-BarringInfo in SIB1 should be set as in the table:

-	Refer to Annex A for the Methods UAC_BarringInfo_xxxxxx() in the tables A1-A3.

Cell B:

NOTE:	This cell is not required for the sequences in Table 5.4.11-1.

-	TAI (MCC/MNC/TAC):	MCC, MNC: see Table 5.4.11-1, TAC: 000002.

-	CellIdentity:	000000002

For Table 5.4.11-1:

uac-BarringInfo in SIB1 should be set as in the table:

-	Refer to Annex A for the Methods UAC_BarringInfo_xxxxxx() in the tables A1-A3.

REGISTRATION ACCEPT IEIs.

-	Operator-Defined Access Category Definitions IEI is configured as defined in Table 5.4.11-1.

-	Refer to Annex A tables A4-A5 for Methods ODAC_definitions1() and ODAC_definitions2()

For HPLMN Cells:

-	Allowed S-NSSAI IEI is configured to include S-NSSAIs '01 01 01 01', '01 01 01 02'and '01 01 01 03'.

For VPLMN Cells:

-	Allowed S-NSSAI IEI is configured to include S-NSSAIs ('01 01 01 01', '01 01 01 01'), ('01 01 01 02', '01 01 01 02'), and ('01 01 01 03', '01 01 01 03').

##### 5.4.11.4.2	Tables related to the test case

Table 5.4.11-1

| TC Seq# | Access | USIM |  | Cell 2 |  | SIB 1: uac-BarringInfo (for all the Cells) | Operator-defined access category definitions IEI in | Result |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | EFUAC_AIC | EFACC (Byte 1 b8-b4) | Cell Id of | PLMN-Identity (MCC/MNC) |  | 1st REGISTRATION ACCEPT, 2nd REGISTRATION ACCEPT | MO Data call 1 successfu? | MO Data call 2 successful? |
| 1.1 | 7,33 | 0x00 00 00 00 | 00000 | Cell A | 246 / 081 | Not Present | ODAC_definitions1(0), ODAC_definitions2(1,16) | Yes | Yes |
| 1.2 | 7,33 | 0x00 00 00 00 | 00000 | Cell A | 246 / 081 | UAC_BarringInfo_Common (33,0x0000000'B) | ODAC_definitions1(0), ODAC_definitions2(1,16) | Yes | No |
| 1.3 | 34,7 | 0x01 00 00 00 | 00000 | Cell A | 246 / 081 | UAC_BarringInfo_Common2 (34,0x1000000'B, 50,0x1000000'B) | ODAC_definitions2(18,1), ODAC_definitions1(2) | No | Yes |
| 1.4 | 7,63 | 0x01 00 00 00 | 00000 | Cell B | 244 / 081 | UAC_BarringInfo_Common2 (63,0x0000000'B, 7,0x0100000'B) | ODAC_definitions1(3), ODAC_definitions2(31,18) | No | No |
| 1.5 | 63,7 | 0x02 00 00 00 | 00000 | Cell A | 246 / 081 | UAC_BarringInfo_Common (63,0x0100000'B) | ODAC_definitions1(31), ODAC - Not present | No | Yes |
| 1.6 | 7,7 | 0x00 00 00 00 | 10000 | Cell B | 246 / 082 | UAC_BarringInfo_Common (48,0x0010000'B) | ODAC_definitions1(16), ODAC - Not present | Yes | Yes |
| 1.7 | 7,7 | 0x00 00 00 00 | 00100 | Cell B | 246 / 082 | UAC_BarringInfo_Common2 (32,0x0000100'B, 34,0x0000100'B) | ODAC_definitions2(0,2), ODAC - Empty | Yes | Yes |

##### 5.4.11.4.3	Procedure

Procedure/steps to be repeated for all sequences listed in table 5.4.11-1:

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | TT | SIB1 of Cell A is transmitted as defined in the initial conditions for tests from table 5.4.11-1 | Barring info in SIB1 is set as in table 5.4.11-1 See Annex A for the Methods UAC_BarringInfo_xxxxxx() in the table |  |  |
| 2 | USER | Power UE on | The UE performs a SIM initialisation. (includes reading of EFUAC-AIC and EFACC) | CR 1 | A.2/1 OR A.2/2 |
| 3 | UE > TT | Send REGISTRATION REQUEST |  |  |  |
| 4 | TT > UE | Send REGISTRATION ACCEPT | 1st REGISTRATION ACCEPT with the Operator-Defined Access Category Definitions IEI as defined in the table 5.4.11-1. For simplicity other signalling is not shown |  |  |

| If 2nd Cell is Cell A as in table 5.4.11-1: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 5 | USER | Disable the UICC profile |  |  |  |
| 6 | USER | Enable the UICC profile |  |  |  |

| If 2nd Cell is Cell B as in table 5.4.11-1: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 5 | USER | Disable the UICC profile | Deactivate Cell A |  |  |
| 6 | USER | Enable the UICC profile | Activate Cell B |  |  |

| Common test steps |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 7 | UE > TT | Send REGISTRATION REQUEST | Send to Cell A or B as in the Table 5.4.11-1 |  |  |
| 8 | USER | Send REGISTRATION ACCEPT | 2nd REGISTRATION ACCEPT with the Operator-Defined Access Category Definitions IEI as defined in the table 5.4.11-1. For simplicity other signalling is not shown |  |  |

| If 1st MO Data call is not allowed: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 9 | UE > TT | Attempt to set up 1st MO Data call | Attempt to set up the MO Data call to DNN TestGp.rs1/S-NSSAI '01 01 01 01' using the MMI or EMMI. | CR 2 to CR 7 |  |

| If 1st MO Data call is allowed: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 9 | UE > TT | Set up MO Data call (PDU SESSION ESTABLISHMENT) | To set up the MO Data call to DNN TestGp.rs1/S-NSSAI '01 01 01 01' using the MMI or EMMI. | CR 2 to CR 7 |  |

| If 2nd MO Data call is not allowed: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 10 | UE > TT | Attempt to set up 2nd MO Data call | Attempt to set up the MO Data call to DNN TestGp.rs2/S-NSSAI '01 01 01 02' using the MMI or EMMI | CR 2 to CR 7 |  |
| 11 | USER | End test sequence |  |  |  |

| If 2nd MO Data call is allowed: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 10 | UE > TT | Set up MO Data call (PDU SESSION ESTABLISHMENT) | To set up the MO Data call to DNN TestGp.rs2/S-NSSAI '01 01 01 02' using the MMI or EMMI. | CR 2 to CR 7 |  |
| 11 | USER | End test sequence |  |  |  |

#### 5.4.11.5	Acceptance criteria

For the scenarios in table 5.4.11-1:

CR 1 is explicitly verified at step 2) by analysing the file READ commands for EFUAC-AIC and EFACC during the USIM-Terminal initialisation procedure.

CR 2, through CR 7 are verified

- at steps 9) by analysing if the UE shall make a successful or not successful 1st MO Data call in accordance with the result indicated in the table if the step is applicable and

- at steps 10) by analysing if the UE shall make a successful or not successful 2nd MO Data call in accordance with the result indicated in the table if the step is applicable.

### 5.4.12	Unified Access Control – Operator-Defined Access Categories, SUPI change

#### 5.4.12.1	Definition and applicability

The purpose of Unified Access Control procedure is to perform access barring check for a 5GS access attempt associated with a given Access Category and one or more Access Identities upon request from upper layers or the RRC layer.

The 5G network shall be able to broadcast barring control information a list of barring parameters associated with an Access Identity and an Access Category in SIB1. Access category numbers in the 32-63 range are used for operator-defined Access Categories.

The EFUAC_AIC in the USIM contains the configuration information pertaining to access identities allocated for high priority services that can be used by the subscriber.

The UE shall be able to determine whether or not a particular new access attempt is allowed based on barring parameters that the UE receives from the broadcast barring control information and the configuration in the USIM.

Operator-defined access category definitions can be signaled to the UE using NAS signaling. Each operator-defined access category definition consists of the parameters a precedence value, an operator-defined access category number, one or more access category criteria type and optionally, a standardized access category.

Operator-defined access category definitions are stored together with a PLMN identity of the PLMN that provided them, and is valid in that PLMN or equivalent PLMN. The operator-defined access category definitions can only be used if the SUPI from the USIM matches the SUPI stored in the non-volatile memory of the ME, else the UE shall delete the operator-defined access category definitions.

In order to determine the access category applicable for the access attempt, the NAS shall check the Access category mapping rules, and use the access category of the lowest rule number for which there is a match for barring check. In the case of operator-defined access categories, the UE shall select the access category from the operator-defined access category definition with the lowest precedence value.

#### 5.4.12.2	Conformance requirement

CR 1	The ME shall read the access control value as part of the USIM initialization procedure, and subsequently adopt this value.

Reference:

-	3GPP TS 31.102 [19], clause 5.1.1.2.

CR 2	Access Identities are configured at the UE as listed in 3GPP TS 22.261 [43] Table 6.22.2.2-1. Access Categories are defined by the combination of conditions related to UE and the type of access attempt as listed in 3GPP TS 22.261 [43] Table 6.22.2.3-1.

Reference:

-	3GPP TS 22.261 [36], clause 6.22.2

CR 3	Operator may provide one or more PLMN-specific Operator-defined access category definitions to the UE using NAS signalling (Operator-defined access category definitions information element), and the UE handles the Operator-defined access category definitions stored for the Registered PLMN, as specified in 3GPP TS 24.501.

References:

-	3GPP TS 23.501 [39], clause 5.2.5

-	3GPP TS 24.501 [25] clause 9.11.3.38

CR 4	Operator-defined access category definitions are stored together with a PLMN identity of the PLMN that provided them, and is valid in that PLMN or equivalent PLMN. The operator-defined access category definitions can only be used if the SUPI from the USIM matches the SUPI stored in the non-volatile memory of the ME, else the UE shall delete the operator-defined access category definitions.

Reference:

-	3GPP TS 24.501 [25], Annex C

CR 5	In order to determine the access category applicable for the access attempt, the NAS shall check the rules in 3GPP TS 24.501 [42] Table 4.5.2.2, and use the access category for which there is a match for barring check. If the access attempt matches more than one rule, the access category of the lowest rule number shall be selected. If the access attempt matches more than one operator-defined access category definition, the UE shall select the access category from the operator-defined access category definition with the lowest precedence value (see 3GPP TS 24.501 [42] clause 4.5.3).

Reference:

-	3GPP TS 24.501 [25], clause 4.5.2

CR 6	If the UE receives Operator-defined access category definitions IE in the REGISTRATION ACCEPT message and the Operator-defined access category definitions IE contains one or more operator-defined access category definitions, the UE shall delete any operator-defined access category definitions stored for the RPLMN and shall store the received operator-defined access category definitions for the RPLMN. If the UE receives the Operator-defined access category definitions IE in the REGISTRATION ACCEPT message and the Operator-defined access category definitions IE contains no operator-defined access category definitions, the UE shall delete any operator-defined access category definitions stored for the RPLMN. If the REGISTRATION ACCEPT message does not contain the Operator-defined access category definitions IE, the UE shall not delete the operator-defined access category definitions stored for the RPLMN.

Reference:

-	3GPP TS 24.501 [25], clause 5.5.1.2.4

CR 7	The UE shall be able to determine whether or not a particular new access attempt is allowed based on uac-BarringInfo broadcast in SIB1.  Access Control check shall be performed as per the information received in uac-BarringInfoSetList.

Reference:

-	3GPP TS 38.331 [37], clauses 5.3.14

#### 5.4.12.3	Test purpose

The purpose of this test is to verify that:

1)	To verify that the ME reads the access control value from EFUAC_AIC and EFACC as part of the USIM-Terminal initialisation procedure, and subsequently adopts this value.

2)	To verify if UE uses operator-defined access categories received within Operator-defined access category definitions information element in the REGISTRATION ACCEPT.

3)	To verify the UE checks the access category applicable for the access attempt, as per the Access category mapping rules and chooses the matching lowest rule.

4)	To verify the UE checks operator-defined access category parameters precedence value, operator-defined access category number and access category criteria type associated with the Operator-defined Access Category when determining the Access Category for the access attempt.

5)	To verify the Operator-defined access category definitions are stored together with a PLMN identity of the PLMN that provided them, and is valid in that PLMN or equivalent PLMN.

6)	To verify the Operator-defined access category definitions can only be used if the SUPI from the USIM matches the SUPI stored in the non-volatile memory of the ME, else the UE shall delete the operator-defined access category definitions.

7)	To verify UE maps the kind of request to one or more access identities and one access category and lower layers performs access barring checks for that request based on the determined access identities and access category.

8)	To verify the UE determines whether or not a particular access attempt is allowed based on uac-BarringInfo broadcast in SIB1 and if the RPLMN is the HPLMN, EHPLMN or visited PLMN of the home country.

#### 5.4.12.4	Method of test

##### 5.4.12.4.1	Initial conditions

The values of the 5G-NR UICC as defined in clause 4.5.9 of the present document are used with the following exceptions:

EFIMSI (International Mobile Subscriber Identity) for UICC 2

Logically:

246081357935799

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 08 | 29 | 64 | 80 | 31 | 75 | 39 | 75 | 99 |

EFUAC_AIC and EFACC

Access Identity is configured in EFUAC_AIC and no Access Classes are configured in EFACC as given in Table 5.4.12-1.

EFUST (USIM Service Table)

Logically:

| Service n°126 |  | UAC Access Identities support | available |
| --- | --- | --- | --- |

Coding:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary: | xxxx xx1x | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxxx xxxx | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | … | B16 | B17 |  |  |
|  | xxxx xxxx | xxxx xxxx | xx11 xxxx | ... | xx10 11xx | 0000 0xxx |  |  |

ME shall be configured with following URSP rules.

Rule Precedence =1

Traffic Descriptor:

DNN=TestGp.rs1

Route Selection Descriptor:

Precedence=1

Network Slice Selection, S-NSSAI: '01 01 01 01' (ST: MBB, SD: 010101)

SSC Mode Selection: SSC Mode 1

Access Type preference: 3GPP access

Rule Precedence = 2

Traffic Descriptor:

DNN=TestGp.rs2

Route Selection Descriptor:

Precedence =1

Network Slice Selection, S-NSSAI: '01 01 01 02' (ST: MBB, SD: 010102)

SSC Mode Selection: SSC Mode 1

The TT (NG-SS) is configured to transmit on the BCCH with the following parameters:

Cell A:

-	MCC, MNC:	246/081

-	TAC:			000001

-	CellIdentity:	000000001

For Table 5.4.12-1:

uac-BarringInfo in SIB1 should be set as in the table:

-	Refer to Annex A for the Methods UAC_BarringInfo_xxxxxx() in the tables A1-A3.

Cell B:

This cell is required for some sequences as in the Table 5.4.12-1.

-	TAI (MCC/MNC/TAC):	MCC, MNC: see Table 5.4.12-1, TAC: 000002.

-	CellIdentity:	000000002

For Table 5.4.12-1:

uac-BarringInfo in SIB1 should be set as in the table:

-	Refer to Annex A for the Methods UAC_BarringInfo_xxxxxx() in the tables A1-A3.

REGISTRATION ACCEPT IEIs.

-	Operator-Defined Access Category Definitions IEI is configured as defined in Table 5.4.12-1.

-	Refer to Annex A tables A4-A5 for Methods ODAC_definitions1() and ODAC_definitions2().

For HPLMN Cells:

-	Allowed S-NSSAI IEI is configured to include S-NSSAIs '01 01 01 01', '01 01 01 02'and '01 01 01 03'.

For VPLMN Cells:

-	Allowed S-NSSAI IEI is configured to include S-NSSAIs ('01 01 01 01', '01 01 01 01'), ('01 01 01 02', '01 01 01 02'), and ('01 01 01 03', '01 01 01 03').

##### 5.4.12.4.2	Tables related to the test case

Table 5.4.12-1

| TC Seq# | Access Category for Call 1 & 2 | USIM |  | Cell 2 |  | SIB 1: uac-BarringInfo (for all the Cells) | Operator-defined access category definitions IEI in | Result |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | EFUAC_AIC | EFACC (Byte 1 b8-b4) | Cell Id of | PLMN-Identity (MCC/MNC) |  | 1st REGISTRATION ACCEPT, 2nd REGISTRATION ACCEPT | MO Data call 1 successfu? | MO Data call 2 successful? |
| 1.1 | 7,33 | 0x00 00 00 00 | 00000 | Cell A | 246 / 081 | Not Present | ODAC_definitions1(0), ODAC_definitions2(1,16) | Yes | Yes |
| 1.2 | 7,33 | 0x00 00 00 00 | 00000 | Cell A | 246 / 081 | UAC_BarringInfo_Common (33,0x0000000'B) | ODAC_definitions1(0), ODAC_definitions2(1,16) | Yes | No |
| 1.3 | 34,7 | 0x01 00 00 00 | 00000 | Cell A | 246 / 081 | UAC_BarringInfo_Common2 (34,0x1000000'B, 50,0x1000000'B) | ODAC_definitions2(18,1), ODAC_definitions1(2) | No | Yes |
| 1.4 | 7,63 | 0x01 00 00 00 | 00000 | Cell B | 244 / 081 | UAC_BarringInfo_Common2 (63,0x0000000'B, 7,0x0100000'B) | ODAC_definitions1(3), ODAC_definitions2(31,18) | No | No |
| 1.5 | 7,7 | 0x02 00 00 00 | 00000 | Cell A | 246 / 081 | UAC_BarringInfo_Common (63,0x0100000'B) | ODAC_definitions1(31), ODAC - Not present | Yes | Yes |
| 1.6 | 7,7 | 0x00 00 00 00 | 10000 | Cell B | 246 / 082 | UAC_BarringInfo_Common (48,0x0010000'B) | ODAC_definitions1(16), ODAC - Not present | Yes | Yes |
| 1.7 | 7,7 | 0x00 00 00 00 | 00100 | Cell B | 246 / 082 | UAC_BarringInfo_Common2 (32,0x0000100'B, 34,0x0000100'B) | ODAC_definitions2(0,2), ODAC - Empty | Yes | Yes |
| 1.8 | 7,7 | 0x00 00 00 00 | 00100 | Cell B | 246 / 082 | UAC_BarringInfo_Common2 (50,0x0000100'B, 33,0x0000100'B) | ODAC_definitions2(18,1), ODAC - Not present | Yes | Yes |

##### 5.4.12.4.3	Procedure

Procedure/steps to be repeated for all sequences listed in table 5.4.12-1:

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | TT | SIB1 of Cell A is transmitted as defined in the initial conditions for tests from table 5.4.12-1 | Barring info in SIB1 is set as in table 5.4.12-1 See Annex A for the Methods UAC_BarringInfo_xxxxxx() in the table |  |  |
| 2 | USER | Power UE on | The UE performs a SIM initialisation. (includes reading of EFUAC-AIC and EFACC) | CR 1 | A.2/1 OR A.2/2 |
| 3 | UE > TT | Send REGISTRATION REQUEST |  |  |  |
| 4 | TT > UE | Send REGISTRATION ACCEPT | 1st REGISTRATION ACCEPT with the Operator-Defined Access Category Definitions IEI as defined in the table 5.4.12-1. For simplicity other signalling is not shown |  |  |

| If 2nd Cell is Cell A as in table 5.4.12-1: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 5 | USER | Disable the UICC profile |  |  |  |
| 6 | USER | Enable the UICC profile |  |  |  |

| If 2nd Cell is Cell B as in table 5.4.12-1: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 5 | USER | Disable the UICC profile | Deactivate Cell A |  |  |
| 6 | USER | Enable the UICC profile | Activate Cell B |  |  |

| Common test steps |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 7 | UE > TT | Send REGISTRATION REQUEST | Send to Cell A or B as in the Table 5.4.12-1 |  |  |
| 8 | USER | Send REGISTRATION ACCEPT | 2nd REGISTRATION ACCEPT with the Operator-Defined Access Category Definitions IEI as defined in the table 5.4.12-1. For simplicity other signalling is not shown |  |  |

| If 1st MO Data call is not allowed: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 9 | UE > TT | Attempt to set up 1st MO Data call | Attempt to set up the MO Data call to DNN TestGp.rs1/S-NSSAI '01 01 01 01' using the MMI or EMMI. | CR 2 to CR 7 |  |

| If 1st MO Data call is allowed: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 9 | UE > TT | Set up MO Data call (PDU SESSION ESTABLISHMENT) | To set up the MO Data call to DNN TestGp.rs1/S-NSSAI '01 01 01 01' using the MMI or EMMI. | CR 2 to CR 7 |  |

| If 2nd MO Data call is not allowed: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 10 | UE > TT | Attempt to set up 2nd MO Data call | Attempt to set up the MO Data call to DNN TestGp.rs2/S-NSSAI '01 01 01 02' using the MMI or EMMI | CR 2 to CR 7 |  |
| 11 | USER | End test sequence |  |  |  |

| If 2nd MO Data call is allowed: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 10 | UE > TT | Set up MO Data call (PDU SESSION ESTABLISHMENT) | To set up the MO Data call to DNN TestGp.rs2/S-NSSAI '01 01 01 02' using the MMI or EMMI. | CR 2 to CR 7 |  |
| 11 | USER | End test sequence |  |  |  |

#### 5.4.12.5	Acceptance criteria

For the Table 5.4.12-1

CR 1 is explicitly verified at step 2) by analysing the file READ commands for EFUAC-AIC and EFACC during the USIM-Terminal initialisation procedure.

CR 2, through CR 7 are verified

- at steps 9) by analysing if the UE shall make a successful or not successful 1st MO Data call in accordance with the result indicated in the table if the step is applicable and

- at steps 10) by analysing if the UE shall make a successful or not successful 2nd MO Data call in accordance with the result indicated in the table if the step is applicable.

## 5.5	Handling of operator controlled features

### 5.5.1	Display of registered 5G PLMN name from USIM

#### 5.5.1.1	Definition and applicability

If the operator’s decision, as indicated by the USIM, is that the ME shall use EFOPL5G in association with EFPNN or EFPNNI to display the Operator 5G PLMN name from USIM, then the ME shall be able to associate the prioritised list of Tracking Area Identity (TAI) identities for NG-RAN in EFOPL5G with the operator name contained in EFPNN. This prioritized list takes precedence over any network name stored within the ME’s internal list and any network name received when registered to the PLMN, as defined by TS 24.501 [25].

#### 5.5.1.2	Conformance requirement

CR 1	EFOPL5G association with the EFPNN shall be performed by the USIM if service n°129 is available in EFUST.

CR 2	The ME shall display the correct Operator network name per 4.4.11.9 in TS 31.102 [19].

Reference:

-	TS 31.102 [19], clauses 4.4.11.9.

-	TS 24.008 [31], clause 10.5.3.5a

#### 5.5.1.3	Test purpose

The purpose of this test is to verify that the ME displays the 5G Operator PLMN name correctly for the following cases:

1)	Entire range of TAC for a specific PLMN is configured in EFOPL5G;

2)	Specific range of TAC for a specific PLMN is configured in EFOPL5G;

3)	Specific TAC for a specific PLMN is configured in EFOPL5G.

#### 5.5.1.4	Method of tests

##### 5.5.1.4.1	Initial conditions

The values of the 5G-NR UICC as defined in clause 4.5.9 of the present document are used with the following exceptions:

EFUST (USIM Service Table)

Logically:

| Service n°42: |  | Operator controlled PLMN selector with Access Technology | available |
| --- | --- | --- | --- |
| Service n°45: |  | PLMN Network Name | available |
| Service n°129: |  | 5GS Operator PLMN List | available |

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary: | xxxx xx1x | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxx1 xx1x | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | … | B16 | B17 |  |  |
|  | xxxx xxxx | xxxx xxxx | xx11 xxxx | ... | xxx0 11xx | xxxx xxx1 |  |  |

EFOPLMNwACT (Operator controlled PLMN selector with Access Technology)

Logically:

1st PLMN:		244 010 (MCC MNC),	1st ACT:	NG-RAN

2nd PLMN:	244 020 (MCC MNC),	2nd ACT:	NG-RAN

3rd PLMN:	244 030 (MCC MNC),	3rd ACT:	NG-RAN

4th PLMN:	244 040 (MCC MNC),	4th ACT:	NG-RAN

5th PLMN:	244 050 (MCC MNC),	5th ACT:	NG-RAN

6th PLMN:	244 060 (MCC MNC),	6th ACT:	NG-RAN

7th PLMN:	244 070 (MCC MNC),	7th ACT:	NG-RAN

8th PLMN:	244 080 (MCC MNC),	8th ACT:	NG-RAN

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 42 | 04 | 10 | 08 | 00 | 42 | 04 | 20 |
|  | B9 | B10 | B11 | B12 | B13 | B14 | B15 | B16 |
|  | 08 | 00 | 42 | 04 | 30 | 08 | 00 | 42 |
|  | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 04 | 40 | 08 | 00 | 42 | 04 | 50 | 08 |
|  | B25 | B26 | B27 | B28 | B29 | B30 | B31 | B32 |
|  | 00 | 42 | 04 | 60 | 08 | 00 | 42 | 04 |
|  | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 |
|  | 70 | 08 | 00 | 42 | 04 | 80 | 08 | 00 |

EFOPL5G

Record 1:

Logically:

MCC:						244,

MNC:						010,

TAC:						Entire range,

PNN Record Identifier:	01

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 42 | 04 | 10 | 00 | 00 | 00 | FF | FF | FE | 01 |

Record 2:

Logically:

MCC:						244,

MNC:						020,

TAC:						000003 - 000006,

PNN Record Identifier:	02

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 42 | 04 | 20 | 00 | 00 | 03 | 00 | 00 | 06 | 02 |

Record 3:

Logically:

MCC:						244,

MNC:						030,

TAC:						000003,

PNN Record Identifier:	02

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 42 | 04 | 30 | 00 | 00 | 03 | 00 | 00 | 03 | 02 |

EFPNN

Record 1:

Logically:

Long name:		PLMN 5G

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 43 | 08 | 87 | 50 | 66 | D3 | 09 | AA | 1D | 01 |
|  | B11 | B12 | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
|  | FF | FF | FF | FF | FF | FF | FF | FF | FF | FF |

Record 2:

Logically:

Long name:		ABCD

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 43 | 05 | 84 | 41 | E1 | 90 | 08 | FF | FF | FF |
|  | B11 | B12 | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
|  | FF | FF | FF | FF | FF | FF | FF | FF | FF | FF |

Record 3:

Logically:

Long name:		CCCDDD

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 43 | 07 | 86 | C3 | E1 | 90 | 48 | 24 | 02 | FF |
|  | B11 | B12 | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
|  | FF | FF | FF | FF | FF | FF | FF | FF | FF | FF |

The NG-SS transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):		244/010/000001

-	Access control:				unrestricted

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and runs an initial activation.

##### 5.5.1.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE > TT | Send REGISTRATION REQUEST | In the registration type IE indicates: - "initial registration" - 5GS mobile identity information element type "SUCI" |  |  |
| 2 | TT > UE | Send REGISTRATION ACCEPT | The REGISTRATION ACCEPT is sent with a 5G-GUTI |  |  |
| 3 | UE > TT | Send REGISTRATION COMPLETE |  |  |  |
| 4 | User | Wait 30 seconds | "PLMN 5G" is shown as Operator 5G PLMN name | CR 1 CR 2 |  |
| 5 | USER/TT | Power off/deactivate the UE |  |  |  |
| 6 | TT | Stop RF output on the BCCH |  |  |  |
| 7 | TT | Resume the RF output on the BCCH with: - TAI (MCC/MNC/TAC): 244/020/000004 - Access control: unrestricted |  |  |  |
| 8 | UE | Runs activation |  |  |  |
| 9 | UE > TT | Send REGISTRATION REQUEST | In the registration type IE indicates: - "initial registration" - 5GS mobile identity information element type "5G-GUTI" |  |  |
| 10 | TT > UE | Send REGISTRATION ACCEPT | The REGISTRATION ACCEPT is sent with a 5G-GUTI |  |  |
| 11 | UE > TT | Send REGISTRATION COMPLETE |  |  |  |
| 12 | User | Wait 30 seconds | "ABCD" is shown as Operator 5G PLMN name | CR 1 CR 2 |  |
| 13 | USER/TT | Power off/deactivate the UE |  |  |  |
| 14 | TT | Stop RF output on the BCCH |  |  |  |
| 15 | TT | Resume the RF output on the BCCH with: - TAI (MCC/MNC/TAC): 244/030/000003 - Access control: unrestricted |  |  |  |
| 16 | USER/TT | Run activation of the UE |  |  |  |
| 17 | UE > TT | Send REGISTRATION REQUEST | In the registration type IE indicates: - "initial registration" - 5GS mobile identity information element type "5G-GUTI" |  |  |
| 18 | TT > UE | Send REGISTRATION ACCEPT | The REGISTRATION ACCEPT is sent with a 5G-GUTI |  |  |
| 19 | UE > TT | Send REGISTRATION COMPLETE |  |  |  |
| 20 | User | Wait 30 seconds | "ABCD" is shown as Operator 5G PLMN name | CR 1 CR 2 |  |
| 21 | USER/TT | Power off/deactivate the UE, then run activation of the UE |  |  |  |

##### 5.5.1.4.3	Acceptance criteria

CR 1 can be implicitly verified. The conformance requirement is met if the expected Operator 5G PLMN name is shown in step 4), step 12) and step 17).

CR 2 shall be verified by checking the UE output for the Operator 5G PLMN name (e.g. on a display). The conformance requirement is met if the appropriate Operator 5G PLMN name is shown:

-	"PLMN 5G" in step 4);

-	"ABCD" in step 12);

-	"ABCD" in step 17).

### 5.5.2	Display of registered 5G PLMN name from ME

#### 5.5.2.1	Definition and applicability

If the operator’s decision, as indicated by the USIM, is that the ME shall use EFOPL5G in association with EFPNN or EFPNNI to display the Operator 5G PLMN name from ME or other sources, then the displayed network name will be either from the one stored within the ME’s internal list or any network name received when registered to the PLMN, as defined by TS 24.501 [25].

#### 5.5.2.2	Conformance requirement

CR 1	EFOPL5G association with the EFPNN shall be performed by the USIM if service n°129 is available in EFUST.

CR 2	The ME shall display the correct Operator network name per 4.4.11.9 in TS 31.102 [19].

Reference:

-	TS 31.102 [19], clauses 4.4.11.9.

-	TS 24.008 [31], clause 10.5.3.5a

#### 5.5.2.3	Test purpose

The purpose of this test is to verify that the ME displays the 5G Operator PLMN name correctly for the following cases:

1)	ME registers to a TAI outside the range referenced in EFOPL5G;

2)	ME registers to a TAI configured in EFOPL5G and PNN record identified is set as 00.

#### 5.5.2.4	Method of tests

##### 5.5.2.4.1	Initial conditions

The values of the 5G-NR UICC as defined in clause 4.5.9 of the present document are used with the following exceptions:

EFUST

Logically:

| Service n°42: |  | Operator controlled PLMN selector with Access Technology | available |
| --- | --- | --- | --- |
| Service n°45: |  | PLMN Network Name | available |
| Service n°129: |  | 5GS Operator PLMN List | available |

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary: | xxxx xx1x | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxx1 xx1x | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | … | B16 | B17 |  |  |
|  | xxxx xxxx | xxxx xxxx | xx11 xxxx | ... | xxx0 11xx | xxxx xxx1 |  |  |

EFOPLMNwACT (Operator controlled PLMN selector with Access Technology)

Logically:

1st PLMN:		244 010 (MCC MNC),	1st ACT:	NG-RAN

2nd PLMN:	244 020 (MCC MNC),	2nd ACT:	NG-RAN

3rd PLMN:	244 030 (MCC MNC),	3rd ACT:	NG-RAN

4th PLMN:	244 040 (MCC MNC),	4th ACT:	NG-RAN

5th PLMN:	244 050 (MCC MNC),	5th ACT:	NG-RAN

6th PLMN:	244 060 (MCC MNC),	6th ACT:	NG-RAN

7th PLMN:	244 070 (MCC MNC),	7th ACT:	NG-RAN

8th PLMN:	244 080 (MCC MNC),	8th ACT:	NG-RAN

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 42 | 04 | 10 | 08 | 00 | 42 | 04 | 20 |
|  | B9 | B10 | B11 | B12 | B13 | B14 | B15 | B16 |
|  | 08 | 00 | 42 | 04 | 30 | 08 | 00 | 42 |
|  | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 04 | 40 | 08 | 00 | 42 | 04 | 50 | 08 |
|  | B25 | B26 | B27 | B28 | B29 | B30 | B31 | B32 |
|  | 00 | 42 | 04 | 60 | 08 | 00 | 42 | 04 |
|  | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 |
|  | 70 | 08 | 00 | 42 | 04 | 80 | 08 | 00 |

EFOPL5G (5GS Operator PLMN List)

Record 1:

Logically:

MCC:						244,

MNC:						010,

TAC:						Entire range,

PNN Record Identifier:	01

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 42 | 04 | 10 | 00 | 00 | 00 | FF | FF | FE | 01 |

Record 2:

Logically:

MCC:						244,

MNC:						020,

TAC:						000003 - 000006,

PNN Record Identifier:	02

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 42 | 04 | 20 | 00 | 00 | 03 | 00 | 00 | 06 | 02 |

Record 3:

Logically:

MCC:						244,

MNC:						030,

TAC:						000005 - 000009,

PNN Record Identifier:	00

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 42 | 04 | 30 | 00 | 00 | 05 | 00 | 00 | 09 | 00 |

EFPNN (PLMN Network Name)

Record 1:

Logically:

Long name:		PLMN 5G

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 43 | 08 | 87 | 50 | 66 | D3 | 09 | AA | 1D | 01 |
|  | B11 | B12 | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
|  | FF | FF | FF | FF | FF | FF | FF | FF | FF | FF |

Record 2:

Logically:

Long name:		ABCD

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 43 | 05 | 84 | 41 | E1 | 90 | 08 | FF | FF | FF |
|  | B11 | B12 | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
|  | FF | FF | FF | FF | FF | FF | FF | FF | FF | FF |

Record 3:

Logically:

Long name:		CCCDDD

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 43 | 07 | 86 | C3 | E1 | 90 | 48 | 24 | 02 | FF |
|  | B11 | B12 | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
|  | FF | FF | FF | FF | FF | FF | FF | FF | FF | FF |

The TT (NG-SS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):		244/020/000007

-	Access control:				unrestricted

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and runs an initial activation.

##### 5.5.2.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE > TT | Send REGISTRATION REQUEST | In the registration type IE indicates: - "initial registration" - 5GS mobile identity information element type "SUCI" |  |  |
| 2 | TT > UE | Send REGISTRATION ACCEPT | The REGISTRATION ACCEPT is sent with a 5G-GUTI |  |  |
| 3 | UE > TT | Send REGISTRATION COMPLETE |  |  |  |
| 4 | USER | Wait 30 seconds | The ME shows: - MCC: "244" - MNC: "020" | CR 1 CR 2 |  |
| 5 | USER/TT | Power off/deactivate the UE |  |  |  |
| 6 | TT | Stop RF output on the BCCH |  |  |  |
| 7 | TT | Resume the RF output on the BCCH with: - TAI (MCC/MNC/TAC): 244/030/000006 - Access control: unrestricted |  |  |  |
| 8 | USER/TT | Run activation of the UE |  |  |  |
| 9 | UE > TT | Send REGISTRATION REQUEST | In the registration type IE indicates: - "initial registration" - 5GS mobile identity information element type "5G-GUTI" |  |  |
| 10 | TT > UE | Send REGISTRATION ACCEPT | The REGISTRATION ACCEPT is sent with a 5G-GUTI |  |  |
| 11 | UE > TT | Send REGISTRATION COMPLETE |  |  |  |
| 12 | USER | Wait 30 seconds | The ME shows: - MCC: "244" - MNC: "030" | CR 1 CR 2 |  |
| 13 | USER/TT | Power off/deactivate the UE |  |  |  |

##### 5.5.1.4.3	Acceptance criteria

CR 1 can be implicitly verified. The conformance requirement is met if the expected or a pre-configured Operator 5G PLMN name is shown in step 4) and step 12).

CR 2 shall be verified by checking the UE output for the Operator 5G PLMN name (e.g. on a display). The conformance requirement is met if the appropriate Operator 5G PLMN name is shown:

-	MCC: "244", MNC: "020", but not: "PLMN 5G", "ABCD" or "CCCDDD" in step 4);

-	MCC: "244", MNC: "030", but not: "PLMN 5G", "ABCD" or "CCCDDD" in step 12);

NOTE:	MCC/MNC combinations are displayed as formatted by the ME manufacturer. MCC/MNC combinations with correct values are valid, independent from the formatting (e.g. with or without a separator). Different from the MCC/MNC combinations anything else configured by ME as Operator 5G PLMN name may be displayed.

## 5.6	Handling subscription identifier privacy for 5G - SUPI type in NAI format

### 5.6.1	SUCI calculation by ME using null scheme

#### 5.6.1.1	Definition and applicability

If the operator's decision is that ME shall calculate the SUCI, the home network operator shall provision a list of the Protection Scheme Identifiers in the USIM that the operator allows. The list of Protection Scheme Identifiers in the USIM may contain one or more Protection Scheme Identifiers in the order of their priority. The ME shall read the SUCI calculation information from the USIM, including the SUPI, the Home Network Public Key, the Home Network Public Key Identifier, and the list of Protection Scheme Identifiers. The ME shall select the protection scheme from its supported schemes that has the highest priority in the list obtained from the USIM.

The ME shall calculate the SUCI using the null-scheme if the highest priority of the protection schemes listed in the USIM is the null-scheme.

#### 5.6.1.2	Conformance requirement

CR 1	SUCI calculation procedure shall be performed by the ME if Service n°124 is available and Service n°125 is not available in EFUST

Reference:

-	TS 31.102 [19], clause 5.3.47.

CR 2	SUPI is available in EFSUPI_NAI if Service n°130 is available in EFUST

Reference:

-	TS 31.102 [19], clauses 4.4.11.10 and 5.3.33.

CR 3	A subscriber identifier is in the form of a SUPI in NAI format

References:

-	TS 31.102 [19], clause 4.4.11.10.

CR 4	The SUPI may contain:

- a NSI, used for private networks as defined in TS 22.261 [36] or

- a GLI and an operator identifier of the 5GC operator, used for supporting FN-BRGs, as further described in TS 23.316 [56] or

- a GCI and an operator identifier of the 5GC operator, used for supporting FN-CRGs and 5G-CRG, as further described in TS 23.316 [56].

References:

-	TS 22.261 [36], clause 8.5;

-	TS 23.316 [56], clause 4.7.3, 4.7.4, 4.7.8 and 4.7.9.

CR 5	As part of the SUCI calculation performed by the ME, the ME performs the reading procedure for EFSUCI_Calc_Info.

Reference:

-	TS 31.102 [19], clauses 4.4.11.8 and 5.3.47.

CR 6	The ME shall calculate the SUCI using the null-scheme if highest priority of the protection schemes listed in the USIM is the null-scheme.

Reference:

-	TS 31.102 [19], clauses 5.3.47 and 5.3.51;

-	TS 33.501 [24], clause Annex C.

#### 5.6.1.3	Test purpose

The purpose of this test is to verify that:

1)	the READ EFSUCI_Calc_Info, EFRouting_Indicator and EFSUPI_NAI commands are performed correctly by the ME.

2)	the ME performs the SUCI calculation procedure using null-scheme.

#### 5.6.1.4	Method of test

##### 5.6.1.4.1	Initial conditions

The values of the 5G-NR UICC – non-IMSI SUPI Type as defined in clause 4.5.10 of the present document are used with the following exception:

EFSUCI_Calc_Info (Subscription Concealed Identifier Calculation Information EF)

Logically:

Protection Scheme Identifier List data object:

Protection Scheme Identifier 1 – null-scheme

Key Index 1: 0

Protection Scheme Identifier 2 – ECIES scheme profile B

Key Index 2: 1

Protection Scheme Identifier 3 – ECIES scheme profile A

Key Index 3: 2

Home Network Public Key List data object:

Home Network Public Key 1 Identifier:27

Home Network Public Key 1:

04 72 DA 71 97 62 34 CE 83 3A 69 07 42 58 67 B8 2E 07 4D 44 EF 90 7D FB 4B 3E 21 C1 C2 25 6E

BC D1 5A 7D ED 52 FC BB 09 7A 4E D2 50 E0 36 C7 B9 C8 C7 00 4C 4E ED C4 F0 68 CD 7B F8 D3

F9 00 E3 B4

Home Network Public Key 2 Identifier: 30

Home Network Public Key 2:

5A 8D 38 86 48 20 19 7C 33 94 B9 26 13 B2 0B 91 63 3C BD 89 71 19 27 3B F8 E4 A6 F4 EE C0 A6

50

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | A0 | 06 | 00 | 00 | 02 | 01 | 01 | 02 |
|  | B9 | B10 | B11 | B12 | B13 | B14 | B15 | B16 |
|  | A1 | 6B | 80 | 01 | 1B | 81 | 41 | 04 |
|  | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 72 | DA | 71 | 97 | 62 | 34 | CE | 83 |
|  | B25 | B26 | B27 | B28 | B29 | B30 | B31 | B32 |
|  | 3A | 69 | 07 | 42 | 58 | 67 | B8 | 2E |
|  | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 |
|  | 07 | 4D | 44 | EF | 90 | 7D | FB | 4B |
|  | B41 | B42 | B43 | B44 | B45 | B46 | B47 | B48 |
|  | 3E | 21 | C1 | C2 | 25 | 6E | BC | D1 |
|  | B49 | B50 | B51 | B52 | B53 | B54 | B55 | B56 |
|  | 5A | 7D | ED | 52 | FC | BB | 09 | 7A |
|  | B57 | B58 | B59 | B60 | B61 | B62 | B63 | B64 |
|  | 4E | D2 | 50 | E0 | 36 | C7 | B9 | C8 |
|  | B65 | B66 | B67 | B68 | B69 | B70 | B71 | B72 |
|  | C7 | 00 | 4C | 4E | ED | C4 | F0 | 68 |
|  | B73 | B74 | B75 | B76 | B77 | B78 | B79 | B80 |
|  | CD | 7B | F8 | D3 | F9 | 00 | E3 | B4 |
|  | B81 | B82 | B83 | B84 | B85 | B86 | B87 | B88 |
|  | 80 | 01 | 1E | 81 | 20 | 5A | 8D | 38 |
|  | B89 | B90 | B91 | B92 | B93 | B94 | B95 | B96 |
|  | 86 | 48 | 20 | 19 | 7C | 33 | 94 | B9 |
|  | B97 | B98 | B99 | B100 | B101 | B102 | B103 | B104 |
|  | 26 | 13 | B2 | 0B | 91 | 63 | 3C | BD |
|  | B105 | B106 | B107 | B108 | B109 | B110 | B111 | B112 |
|  | 89 | 71 | 19 | 27 | 3B | F8 | E4 | A6 |
|  | B113 | B114 | B115 | B116 | B117 |
|  | F4 | EE | C0 | A6 | 50 |

The TT (NG-SS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):		244/083/000001

-	Access control:				unrestricted.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and runs an initial activation.

##### 5.6.1.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | READ EFUST | (Evaluation of service settings) | (CR 1) (CR 2) |  |
| 2 | UE | READ EFSUPI_NAI, EFSUCI_Calc_Info, EFRouting_indicator |  | CR 2 (CR 3) CR 4 CR 5 | A.2/1 OR A.2/2 |
| 3 | ME | Perform SUCI calculation | The ME performs a SUCI calculation using null scheme | (CR 1)(CR 6) |  |
| 4 | UE > TT | Send REGISTRATION REQUEST | The UE sends a REGISTRATION REQUEST with 5GS registration type IE as "initial registration" and 5GS mobile identity information element type "SUCI" | CR 1 CR 3 CR 6 |  |
| 5 | TT > UE | Send REGISTRATION ACCEPT | The TT sends a REGISTRATION ACCEPT with 5G-GUTI |  |  |
| 6 | UE > TT | Send REGISTRATION COMPLETE |  | CR 4 |  |

| Optional step if CR 2, CR 4 and CR 5 verification cannot be performed at step 2) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 7 | UE | READ EFSUPI_NAI, EFSUCI_Calc_Info, EFRouting_indicator |  | CR 2 CR 3 CR 5 | A.2/3 |

#### 5.6.1.5	Acceptance criteria

CR 1, CR 3 and CR 6 are implicitly verified in step 4). The conformance requirements are met if the 5GS mobile identity IE in the REGISTRATION REQUEST performed in step 4) includes the following values:

-	SUPI format:						2

-	NAI format for the SUCI:		type3.rid17.schid0.userid00-00-5E-00-53-00@5gc.mnc012.mcc345.3gppnetwork.org

-	SUPI Type:						3

-	Home Network Identifier:		5gc.mnc012.mcc345.3gppnetwork.org

-	Routing indicator:				17

-	Protection scheme ID:			00

-	Home network public key ID:	0

-	Scheme output:					00-00-5E-00-53-00

CR 2, CR 4 and CR 5 can be verified by a method explicitly verifying the correct execution of the READ commands on the listed EFs. The CRs are met if the READ procedure on EFSUCI_Calc_Info, EFRouting_indicator and EFSUPI_NAI has been executed as defined in [28], clause 14.1.1. This either is done with methods A.2/1 or A.2/2 in step 2) or in the optional step 7) with method A.2/3.

CR 4 is implicitly verified in step 6). The requirement is met if the registration can be completed successfully.

### 5.6.2	UE identification by SUCI during initial registration – SUCI calculation by USIM using profile A

#### 5.6.2.1	Definition and applicability

If the operator's decision, indicated by the USIM, is that the USIM shall calculate the SUCI, then the USIM shall not give to the ME any parameter for the calculation of the SUCI including the Home Network Public Key Identifier, the Home Network Public Key, and the Protection Scheme Identifier. If the ME determines that the calculation of the SUCI, indicated by the USIM, shall be performed by the USIM, the ME shall delete any previously received or locally cached parameters for the calculation of the SUCI including the Routing Indicator, the Home Network Public Key Identifier, the Home Network Public Key and the Protection Scheme Identifier.

#### 5.6.2.2	Conformance requirement

CR 1	SUCI calculation procedure shall be performed by the USIM if Services n°124 and n°125 are available in EFUST

Reference:

-	TS 31.102 [19], clause 5.3.47.

CR 2	SUPI is available in EFSUPI_NAI if Service n°130 is available in EFUST

Reference:

-	TS 31.102 [19], clauses 4.4.11.10 and 5.3.33.

CR 3	A subscriber identifier is in the form of a SUPI in NAI format

References:

-	TS 31.102 [19], clause 4.4.11.10.

CR 4	The SUPI may contain:

- a NSI, used for private networks as defined in TS 22.261 [36] or

- a GLI and an operator identifier of the 5GC operator, used for supporting FN-BRGs, as further described in TS 23.316 [56] or

- a GCI and an operator identifier of the 5GC operator, used for supporting FN-CRGs and 5G-CRG, as further described in TS 23.316 [56].

References:

-	TS 22.261 [36], clause 8.5;

-	TS 23.316 [56], clause 4.7.3, 4.7.4, 4.7.8 and 4.7.9.

CR 5	The ME shall use the GET IDENTITY command in SUCI context to retrieve the SUCI calculated by the USIM.

Reference:

-	TS 31.102 [19], clause 5.3.48.

CR 6	This GET IDENTITY command shall be as per 7.5.2 in 3GPP TS 31.102 [19].

Reference:

-	TS 31.102 [19], clause 7.5.2.

CR 7	The USIM shall calculate the SUCI using the ECIES scheme profile A.

Reference:

-	TS 31.102 [19], clauses 5.3.48;

-	TS 33.501 [24], clause Annex C.

#### 5.6.2.3	Test purpose

The purpose of this test is to verify that:

1)	that the GET IDENTITY command is performed correctly by the ME.

2)	the ME includes the SUCI received from the USIM within the GET IDENTITY response in the 5GS mobile identity IE.

#### 5.6.2.4	Method of test

##### 5.6.2.4.1	Initial conditions

The values of the 5G-NR UICC – non-IMSI SUPI Type as defined in clause 4.5.10 of the present document are used with the following exception:

EFUST (USIM Service Table)

Services defined in Releases higher than Rel-16 might be available.

Logically:

| Service n°125: |  | SUCI calculation by the USIM | available |
| --- | --- | --- | --- |

Coding:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary: | xx1x xx11 | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxxx xxxx | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | … | B16 | B17 |  |  |
|  | xxxx xxxx | xxxx xxxx | xx11 xxxx | ... | xxx1 111x | xxxx xxxx |  |  |

EFSUPI_NAI (SUPI as Network Access Identifier)

Logically:			verylongusername1@3gpp.com

SUPI Type:	NSI

Username:	verylongusername1

Realm:		3gpp.com

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 80 | 1A | 76 | 65 | 72 | 79 | 6C | 6F | 6E | 67 | 75 | 73 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 65 | 72 | 6E | 61 | 6D | 65 | 31 | 40 | 33 | 67 | 70 | 70 |
|  | B25 | B26 | B27 | B28 |
|  | 2E | 63 | 6F | 6D |

5G-NR UICC is configured with:

Protection Scheme Identifier:				ECIES scheme profile A

Key Index:									1

Home Network Public Key Identifier:	30

Home Network Public Key:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 5A | 8D | 38 | 86 | 48 | 20 | 19 | 7C | 33 | 94 | B9 | 26 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 13 | B2 | 0B | 91 | 63 | 3C | BD | 89 | 71 | 19 | 27 | 3B |
|  | B25 | B26 | B27 | B28 | B29 | B30 | B31 | B32 |
|  | F8 | E4 | A6 | F4 | EE | C0 | A6 | 50 |

EFSUCI_Calc_Info (Subscription Concealed Identifier Calculation Information EF):	Not available to the ME.

The TT (NG-SS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):		244/083/000001

-	Access control:				unrestricted.

The NG-SS shall be configured with Home Network Private Key for profile A:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | C5 | 3C | 22 | 20 | 8B | 61 | 86 | 0B | 06 | C6 | 2E | 54 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 06 | A7 | B3 | 30 | C2 | B5 | 77 | AA | 55 | 58 | 98 | 15 |
|  | B25 | B26 | B27 | B28 | B29 | B30 | B31 | B32 |
|  | 10 | D1 | 28 | 24 | 7D | 38 | BD | 1D |

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and runs an initial activation.

##### 5.6.2.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | READ EFUST, EFSUPI_NAI | (Evaluation of service settings) | (CR 1) (CR 2) (CR 3) |  |
| 2 | ME > TT | Send GET IDENTITY | The ME sends a GET IDENTITY command with Identity Context in P2 as SUCI (0x01) to the 5G-NR UICC | CR 5 CR 6 | A.2/1 OR A.2/2 |
| 2a | USIM | Perform SUCI calculation | The USIM shall calculate the SUCI using the ECIES scheme profile A | (CR 7) |  |
| 3 | UE > TT | Send REGISTRATION REQUEST | The UE sends a REGISTRATION REQUEST with 5GS registration type IE as "initial registration" and 5GS mobile identity information element type "SUCI" | CR 1 CR 2 CR 3 CR 7 |  |
| 4 | TT > UE | Send REGISTRATION ACCEPT | The TT sends a REGISTRATION ACCEPT with 5G-GUTI |  |  |
| 5 | UE > TT | Send REGISTRATION COMPLETE |  | CR 4 |  |

#### 5.6.2.5	Acceptance criteria

CR 1, CR 2, CR 3 and CR 7 are implicitly verified in step 3). The conformance requirements are met if the 5GS mobile identity IE in the REGISTRATION REQUEST performed in step 3) includes the following values:

-	SUPI format:						1

-	NAI format for the SUCI:		type1.rid17.schid1.hnkey30.ecckey<ECC ephemeral public key>.cip<encryption of "verylongusername1">.mac<MAC tag value>@3gpp.com

-	SUPI Type:						1

-	Home Network Identifier:		3gpp.com

-	Routing indicator:				17

-	Protection scheme ID:			01

-	Home network public key ID:	30

-	Scheme output:					ECC ephemeral public key, encryption of "verylongusername1" and MAC tag value

CR 5 and CR 6 can be verified by a method explicitly verifying the correct execution and contents of the GET IDENTITY command (A.2/1 or A.2/2).

CR 4 is met if the UE sends REGISTRATION COMPLETE message to the TT (NG-SS) in step 5).

CR 1, CR 2, CR 3 may optionally be verified in step 1) using explicit verification (A.2/1, A.2/2).

CR 7 may optionally be verified in step 2a) using explicit verification (A.2/1, A.2/2).

Example with test data from 3GPP TS 33.501 [43] Annex C:

type1.rid17.schid1.hnkey30.ecckey977D8B2FDAA7B64AA700D04227D5B440630EA4EC50F9082273A26BB678C92222.cip8E358A1582ADB15322C10E515141D2039A.mac12E1D7783A97F1AC@3gpp.com

### 5.6.3	UE identification by SUCI during initial registration – SUCI calculation by USIM using profile B

#### 5.6.3.1	Definition and applicability

If the operator's decision, indicated by the USIM, is that the USIM shall calculate the SUCI, then the USIM shall not give to the ME any parameter for the calculation of the SUCI including the Home Network Public Key Identifier, the Home Network Public Key, and the Protection Scheme Identifier. If the ME determines that the calculation of the SUCI, indicated by the USIM, shall be performed by the USIM, the ME shall delete any previously received or locally cached parameters for the calculation of the SUCI including the Routing Indicator, the Home Network Public Key Identifier, the Home Network Public Key and the Protection Scheme Identifier.

#### 5.6.3.2	Conformance requirement

CR 1	SUCI calculation procedure shall be performed by the USIM if Services n°124 and n°125 are available in EFUST

Reference:

-	TS 31.102 [19], clause 5.3.47.

CR 2	SUPI is available in EFSUPI_NAI if Service n°130 is available in EFUST

Reference:

-	TS 31.102 [19], clauses 4.4.11.10 and 5.3.33.

CR 3	A subscriber identifier is in the form of a SUPI in NAI format

References:

-	TS 31.102 [19], clause 4.4.11.10.

CR 4	The SUPI may contain:

- a NSI, used for private networks as defined in TS 22.261 [36] or

- a GLI and an operator identifier of the 5GC operator, used for supporting FN-BRGs, as further described in TS 23.316 [56] or

- a GCI and an operator identifier of the 5GC operator, used for supporting FN-CRGs and 5G-CRG, as further described in TS 23.316 [56].

References:

-	TS 22.261 [36], clause 8.5;

-	TS 23.316 [56], clause 4.7.3, 4.7.4, 4.7.8 and 4.7.9.

CR 5	The ME shall use the GET IDENTITY command in SUCI context to retrieve the SUCI calculated by the USIM.

Reference:

-	TS 31.102 [19], clause 5.3.48.

CR 6	This GET IDENTITY command shall be as per 7.5.2 in 3GPP TS 31.102 [19].

Reference:

-	TS 31.102 [19], clause 7.5.2.

CR 7	The USIM shall calculate the SUCI using the ECIES scheme profile B.

Reference:

-	TS 31.102 [19], clauses 5.3.48;

-	TS 33.501 [24], clause Annex C.

#### 5.6.3.3	Test purpose

The purpose of this test is to verify that:

1)	that the GET IDENTITY command is performed correctly by the ME.

2)	the ME includes the SUCI received from the USIM within the GET IDENTITY response in the 5GS mobile identity IE.

#### 5.6.3.4	Method of test

##### 5.6.3.4.1	Initial conditions

The values of the 5G-NR UICC – non-IMSI SUPI Type as defined in clause 4.5.10 of the present document are used with the following exception:

EFUST (USIM Service Table)

Services defined in Releases higher than Rel-16 might be available.

Logically:

| Service n°125: |  | SUCI calculation by the USIM | available |
| --- | --- | --- | --- |

Coding:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary: | xx1x xx11 | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxxx xxxx | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | … | B16 | B17 |  |  |
|  | xxxx xxxx | xxxx xxxx | xx11 xxxx | ... | xxx1 111x | xxxx xxxx |  |  |

EFSUPI_NAI (SUPI as Network Access Identifier)

Logically:			verylongusername1@3gpp.com

SUPI Type:	NSI

Username:	verylongusername1

Realm:		3gpp.com

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 80 | 1A | 76 | 65 | 72 | 79 | 6C | 6F | 6E | 67 | 75 | 73 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 65 | 72 | 6E | 61 | 6D | 65 | 31 | 40 | 33 | 67 | 70 | 70 |
|  | B25 | B26 | B27 | B28 |
|  | 2E | 63 | 6F | 6D |

5G-NR UICC is configured with:

Protection Scheme Identifier:				ECIES scheme profile B

Key Index:									1

Home Network Public Key Identifier:	27

Home Network Public Key:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 04 | 72 | DA | 71 | 97 | 62 | 34 | CE | 83 | 3A | 69 | 07 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 42 | 58 | 67 | B8 | 2E | 07 | 4D | 44 | EF | 90 | 7D | FB |
|  | B25 | B26 | B27 | B28 | B29 | B30 | B31 | B32 | B33 | B34 | B35 | B36 |
|  | 4B | 3E | 21 | C1 | C2 | 25 | 6E | BC | D1 | 5A | 7D | ED |
|  | B37 | B38 | B39 | B40 | B41 | B42 | B43 | B44 | B45 | B46 | B47 | B48 |
|  | 52 | FC | BB | 09 | 7A | 4E | D2 | 50 | E0 | 36 | C7 | B9 |
|  | B49 | B50 | B51 | B52 | B53 | B54 | B55 | B56 | B57 | B58 | B59 | B60 |
|  | C8 | C7 | 00 | 4C | 4E | ED | C4 | F0 | 68 | CD | 7B | F8 |
|  | B61 | B62 | B63 | B64 | B65 |
|  | D3 | F9 | 00 | E3 | B4 |

EFSUCI_Calc_Info (Subscription Concealed Identifier Calculation Information EF):	Not available to the ME.

The TT (NG-SS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):		244/083/000001

-	Access control:				unrestricted.

The NG-SS shall be configured with Home Network Private Key for profile B:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | F1 | AB | 10 | 74 | 47 | 7E | BC | C7 | F5 | 54 | EA | 1C |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 5F | C3 | 68 | B1 | 61 | 67 | 30 | 15 | 5E | 00 | 41 | AC |
|  | B25 | B26 | B27 | B28 | B29 | B30 | B31 | B32 |
|  | 44 | 7D | 63 | 01 | 97 | 5F | EC | DA |

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and runs an initial activation.

##### 5.6.3.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | READ EFUST, EFSUPI_NAI | (Evaluation of service settings) | (CR 1) (CR 2) (CR 3) |  |
| 2 | ME > TT | Send GET IDENTITY | The ME sends a GET IDENTITY command with Identity Context in P2 as SUCI (0x01) to the 5G-NR UICC | CR 5 CR 6 | A.2/1 OR A.2/2 |
| 2a | USIM | Perform SUCI calculation | The USIM shall calculate the SUCI using the ECIES scheme profile B | (CR 7) |  |
| 3 | UE > TT | Send REGISTRATION REQUEST | The UE sends a REGISTRATION REQUEST with 5GS registration type IE as "initial registration" and 5GS mobile identity information element type "SUCI" | CR 1 CR 2 CR 3 CR 7 |  |
| 4 | TT > UE | Send REGISTRATION ACCEPT | The TT sends a REGISTRATION ACCEPT with 5G-GUTI |  |  |
| 5 | UE > TT | Send REGISTRATION COMPLETE |  | CR 4 |  |

#### 5.6.3.5	Acceptance criteria

CR 1, CR 2, CR 3 and CR 7 are implicitly verified in step 3). The conformance requirements are met if the 5GS mobile identity IE in the REGISTRATION REQUEST performed in step 3) includes the following values:

-	SUPI format:						1

-	NAI format for the SUCI:		type1.rid17.schid2.hnkey27.ecckey<ECC ephemeral public key>.cip<encryption of "verylongusername1">.mac<MAC tag value>@3gpp.com

-	SUPI Type:						1

-	Home Network Identifier:		3gpp.com

-	Routing indicator:				17

-	Protection scheme ID:			02

-	Home network public key ID:	27

-	Scheme output:					ECC ephemeral public key, encryption of "verylongusername1" and MAC tag value

CR 5 and CR 6 can be verified by a method explicitly verifying the correct execution and contents of the GET IDENTITY command (A.2/1 or A.2/2).

CR 4 is met if the UE sends REGISTRATION COMPLETE message to the TT (NG-SS) in step 5).

CR 1, CR 2, CR 3 may optionally be verified in step 1) using explicit verification (A.2/1, A.2/2).

CR 7 may optionally be verified in step 2a) using explicit verification (A.2/1, A.2/2).

Example with test data from 3GPP TS 33.501 [43] Annex C:

type1.rid17.schid2.hnkey27.ecckey03759BB22C563D9F4A6B3C1419E543FC2F39D6823F02A9D71162B39399218B244B.cipBE22D8B9F856A52ED381CD7EAF4CF2D525.mac3CDDC61A0A7882EB@3gpp.com

### 5.6.4	UE identification after SUPI is changed

#### 5.6.4.1	Definition and applicability

A globally unique temporary user identity for 5GS-based services, the 5G globally unique temporary identity (5G-GUTI), is used for identification within the signalling procedures. A UE supporting N1 mode includes a valid 5G-GUTI, if any is available, in the REGISTRATION REQUEST and DEREGISTRATION REQUEST messages.

#### 5.6.4.2	Conformance requirement

CR 1	The following 5GMM parameters shall be stored on the USIM if the corresponding file is present:

-	5G-Globally Unique Temporary Identifier (5G-GUTI);

-	Last visited registered Tracking Area Identity in 5GS (TAI);

-	5GS update status;

-	5G NAS security context parameters from a full native 5G NAS security context.

NOTE:	The presence and format of corresponding files on the USIM is specified in 3GPP TS 31.102 [4].

Reference:

-	TS 31.102 [19], clauses 4.4.11.2

CR 2	If the corresponding file is not present on the USIM, these 5GMM parameters are stored in a non-volatile memory in the ME together with the SUPI from the USIM (EFSUPI_NAI). These 5GMM parameters can only be used if the SUPI from the USIM matches the SUPI stored in the non-volatile memory; else the UE shall delete the 5GMM parameters.

Reference:

-	TS 24.501 [25], clauses 5.3.3, 5.5.1.2 and Annex C.

CR 3	The ME correctly performs the READ BINARY command on EFSUPI_NAI.

References:

-	TS 31 101 [33], clause 11.1.3;

-	ETSI TS 102 221 [8], clause 11.1.3 and 14.1.1.

CR 4	The ME shall use the GET IDENTITY command to retrieve the SUCI calculated by the USIM.

Reference:

-	TS 31.102 [19], clause 5.3.48.

#### 5.6.4.3	Test purpose

The purpose of this test is to verify that:

- the READ EFSUPI_NAI command is performed correctly by the ME.

- the ME deletes the 5GMM parameters from non-volatile memory in case SUPI is changed.

- the GET IDENTITY command is performed correctly by the ME.

- the ME includes the SUCI received from the USIM within GET IDENTITY response in the 5GS mobile identity IE.

#### 5.6.4.4	Method of test

##### 5.6.4.4.1	Initial conditions

The values of the 5G-NR UICC – non-IMSI SUPI Type as defined in clause 4.5.10 of the present document are used with the following exception:

EFUST (USIM Service Table)

Services defined in Releases higher than Rel-16 might be available.

Logically:

| Service n°125: |  | SUCI calculation by the USIM | available |
| --- | --- | --- | --- |

Coding:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary: | xx1x xx11 | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxxx xxxx | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | … | B16 | B17 |  |  |
|  | xxxx xxxx | xxxx xxxx | xx11 xxxx | ... | xxx1 111x | xxxx xxxx |  |  |

5G-NR UICC is configured with:

Protection Scheme Identifier:				null-scheme

Key Index:									0

EFSUCI_Calc_Info (Subscription Concealed Identifier Calculation Information EF):	Not available to the ME.

The TT (NG-SS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):		244/083/000001

-	Access control:				unrestricted.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and runs an initial activation.

##### 5.6.4.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | READ EFUST, EFSUPI_NAI | Evaluation of service settings. Verification of the READ command on EFSUPI_NAI | (CR 1) (CR 2) CR 3 | A.2/1 OR A.2/2 |
| 2 | ME > TT | Send GET IDENTITY | The ME sends a GET IDENTITY command with Identity Context in P2 as SUCI (0x01) to the 5G-NR UICC | CR 4 | A.2/1 OR A.2/2 |
| 3 | UE > TT | Send REGISTRATION REQUEST | The UE sends a REGISTRATION REQUEST with 5GS registration type IE as "initial registration" |  |  |
| 4 | TT > UE | Send REGISTRATION ACCEPT | The REGISTRATION ACCEPT sent by the TT contains: - 5G-GUTI: "24408300010266436587" - 5GS TAI list with TAI with: - TAI: 244/083/000001 | CR 1 CR 2 |  |
| 5 | UE > TT | Send REGISTRATION COMPLETE |  |  |  |
| 6 | USER/TT | Power off/deactivate the UE, then run activation of the UE | Before power on the SUPI value is set to: 00-00-5E-00-53-00@5gc.mnc012.mcc345.3gppnetwork.org to 00-00-5E-00-53-01@5gc.mnc012.mcc345.3gppnetwork.org |  |  |
| 7 | UE | READ EFSUPI_NAI | Verification of the READ command on EFSUPI_NAI | CR 3 | A.2/1 OR A.2/2 |
| 8 | ME > TT | Send GET IDENTITY | The ME sends a GET IDENTITY command with Identity Context in P2 as SUCI (0x01) to the 5G-NR UICC | CR 4 | A.2/1 OR A.2/2 |
| 9 | UE > TT | Send REGISTRATION REQUEST | The UE sends a REGISTRATION REQUEST with 5GS registration type IE as "initial registration" and 5GS mobile identity information element type "SUCI" with the new SUCI corresponding to the new SUPI value | CR 1 CR 2 |  |

#### 5.6.4.5	Acceptance criteria

CR 1 and CR 2 are implicitly verified in step 5). The conformance requirements are met if the REGISTRATION ACCEPT performed in step 4) contains:

-	5G GUTI:			24408300010266436587

-	TAI:				244/083/000001

and the REGISTRATION REQUEST performed in step 10) includes the new SUCI:

-	SUPI format:						2

-	NAI format for the SUCI:		type3.rid17.schid0.userid00-00-5E-00-53-01@5gc.mnc012.mcc345.3gppnetwork.org

-	SUPI Type:						3

-	Home Network Identifier:		5gc.mnc012.mcc345.3gppnetwork.org

-	Routing indicator:				17

-	Protection scheme ID:			00

-	Home network public key ID:	0

-	Scheme output:					00-00-5E-00-53-01

CR 3 can be explicitly verified. The requirement is met if the READ command on EFSUPI_NAI is correctly performed in step 1) and step 7) (A.2/1 or A.2/2).

CR 4 can be verified by a method explicitly verifying the correct execution and contents of the GET IDENTITY command in step 2) and step 8) (A.2/1 or A.2/2).

CR 1 and CR 2 may optionally be verified in step 1) using explicit verification (A.2/1, A.2/2).

### 5.6.5	UE identification by SUCI during initial registration – SUCI calculation by ME using profile A

#### 5.6.5.1	Definition and applicability

If the operator's decision is that the ME shall calculate the SUCI, the Home Network Operator shall provision a list of the Protection Scheme Identifiers that the operator allows in the USIM. The list of Protection Scheme Identifiers in the USIM may contain one or more Protection Scheme Identifiers in order of their priority. The ME shall read the SUCI calculation information from the USIM, including the SUPI, the Home Network Public Key, the Home Network Public Key Identifier, and the list of Protection Scheme Identifiers. The ME shall select the protection scheme from its supported schemes that has the highest priority in the list obtained from the USIM.

#### 5.6.5.2	Conformance requirement

CR 1	SUCI calculation procedure shall be performed by the ME if Service n°124 is available and Service n°125 is not available in EFUST

Reference:

-	TS 31.102 [19], clause 5.3.47.

CR 2	SUPI is available in EFSUPI_NAI if Service n°130 is available in EFUST

Reference:

-	TS 31.102 [19], clauses 4.4.11.10, 7.5.2 and Annex N.

CR 3	A subscriber identifier is in the form of a SUPI in NAI format

References:

-	TS 31.102 [19], clause 4.4.11.10.

CR 4	The SUPI may contain:

- a NSI, used for private networks as defined in TS 22.261 [36] or

- a GLI and an operator identifier of the 5GC operator, used for supporting FN-BRGs, as further described in TS 23.316 [56] or

- a GCI and an operator identifier of the 5GC operator, used for supporting FN-CRGs and 5G-CRG, as further described in TS 23.316 [56].

References:

-	TS 22.261 [36], clause 8.5;

-	TS 23.316 [56], clause 4.7.3, 4.7.4, 4.7.8 and 4.7.9.

CR 5	As part of the SUCI calculation performed by the ME, the ME performs the reading procedure for EFSUCI_Calc_Info.

Reference:

-	TS 31.102 [19], clause 4.11.8 and 5.3.47.

CR 6		The ME shall calculate the SUCI using the ECIES scheme profile A if highest priority of the protection schemes listed in the USIM is the ECIES scheme profile A.

Reference:

-	TS 31.102 [19], clauses 5.3.47;

-	TS 33.501 [24], clause Annex C.

#### 5.6.5.3	Test purpose

The purpose of this test is to verify that:

1)	that the READ EFSUCI_Calc_Info, EFRouting_Indicator and EFSUPI_NAI commands are performed correctly by the ME;

2)	the ME performs SUCI calculation procedure using the profile with the highest priority (i.e. ECIES scheme profile A and the Home Network Public Key).

#### 5.6.5.4	Method of test

##### 5.6.5.4.1	Initial conditions

The values of the 5G-NR UICC – non-IMSI SUPI Type as defined in clause 4.5.10 of the present document are used with the following exception:

EFSUCI_Calc_Info (Subscription Concealed Identifier Calculation Information EF)

Logically:

Protection Scheme Identifier List data object:

Protection Scheme Identifier 1 – ECIES scheme profile A

Key Index 1:		1

Protection Scheme Identifier 2 – ECIES scheme profile B

Key Index 2:		2

Protection Scheme Identifier 3 – null-scheme

Key Index 3:		0

Home Network Public Key List data object

Home Network Public Key 1 Identifier:	30

Home Network Public Key 1:

-	5A 8D 38 86 48 20 19 7C 33 94 B9 26 13 B2 0B 91 63 3C BD 89 71 19 27 3B F8 E4 A6 F4 EE C0 A6 50

Home Network Public Key 2 Identifier:	27

Home Network Public Key 2:

-	04 72 DA 71 97 62 34 CE 83 3A 69 07 42 58 67 B8 2E 07 4D 44 EF 90 7D FB 4B 3E 21 C1 C2 25 6E BC D1 5A 7D ED 52 FC BB 09 7A 4E D2 50 E0 36 C7 B9 C8 C7 00 4C 4E ED C4 F0 68 CD 7B F8 D3 F9 00 E3 B4

EFSUPI_NAI (SUPI as Network Access Identifier)

Logically:			verylongusername1@3gpp.com

SUPI Type:	NSI

Username:	verylongusername1

Realm:		3gpp.com

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 80 | 1A | 76 | 65 | 72 | 79 | 6C | 6F | 6E | 67 | 75 | 73 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 65 | 72 | 6E | 61 | 6D | 65 | 31 | 40 | 33 | 67 | 70 | 70 |
|  | B25 | B26 | B27 | B28 |
|  | 2E | 63 | 6F | 6D |

The TT (NG-SS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):		244/083/000001

-	Access control:				unrestricted.

The NG-SS shall be configured with Home Network Private Key for profile A:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | C5 | 3C | 22 | 20 | 8B | 61 | 86 | 0B | 06 | C6 | 2E | 54 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 06 | A7 | B3 | 30 | C2 | B5 | 77 | AA | 55 | 58 | 98 | 15 |
|  | B25 | B26 | B27 | B28 | B29 | B30 | B31 | B32 |
|  | 10 | D1 | 28 | 24 | 7D | 38 | BD | 1D |

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and runs an initial activation.

##### 5.6.5.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | READ EFUST, EFSUPI_NAI, EFRouting_Indicator and EFSUCI_Calc_Info | (Evaluation of service settings) Verification of the READ command and the EF contents | (CR 1) (CR 2) CR 5 | A.2/1 OR A.2/2 |
| 1a | ME | Perform SUCI calculation | The ME shall calculate the SUCI using the ECIES scheme profile A | (CR 6) |  |
| 2 | UE > TT | Send REGISTRATION REQUEST | The UE sends a REGISTRATION REQUEST with 5GS registration type IE as "initial registration" and 5GS mobile identity information element type "SUCI" | CR 1 CR 2 CR 3 CR 4 CR 6 |  |
| 3 | TT > UE | Send REGISTRATION ACCEPT | The TT sends a REGISTRATION ACCEPT with 5G-GUTI |  |  |
| 4 | UE > TT | Send REGISTRATION COMPLETE |  |  |  |

#### 5.6.5.5	Acceptance criteria

CR 1, CR 2, CR 3, CR 4 and CR 6 are implicitly verified in step 2). The conformance requirements are met if the 5GS mobile identity IE in the REGISTRATION REQUEST performed in step 2) includes the following values:

-	SUPI format:						1

-	NAI format for the SUCI:		type1.rid17.schid1.hnkey30.ecckey<ECC ephemeral public key>.cip<encryption of "verylongusername1">.mac<MAC tag value>@3gpp.com

-	SUPI Type:						1

-	Home Network Identifier:		3gpp.com

-	Routing indicator:				17

-	Protection scheme ID:			01

-	Home network public key ID:	30

-	Scheme output:					ECC ephemeral public key, encryption of "verylongusername1" and MAC tag value

CR 1 and CR 2 may be explicitly verified in step 1) if the READ commands on EFUST and EFSUPI_NAI are analysed using either of the methods A.2/1 or A.2/2. CRs for explicit contents verification of EFRouting_Indicator and EFSUCI_Calc_Info are not defined.

CR 5 is explicitly verified in step 1). The requirement is met if EFSUCI_Calc_Info is read (A.2/1 or A.2/2).

CR 6 may optionally be explicitly verified in step 1a) by any method tracing the ME behaviour.

Example with test data from 3GPP TS 33.501 [43] Annex C:

type1.rid17.schid1.hnkey30.ecckey977D8B2FDAA7B64AA700D04227D5B440630EA4EC50F9082273A26BB678C92222.cip8E358A1582ADB15322C10E515141D2039A.mac12E1D7783A97F1AC@3gpp.com

### 5.6.6	UE identification by SUCI during initial registration – SUCI calculation by ME using profile A

#### 5.6.6.1	Definition and applicability

If the operator's decision is that the ME shall calculate the SUCI, the Home Network Operator shall provision a list of the Protection Scheme Identifiers that the operator allows in the USIM. The list of Protection Scheme Identifiers in the USIM may contain one or more Protection Scheme Identifiers in order of their priority. The ME shall read the SUCI calculation information from the USIM, including the SUPI, the Home Network Public Key, the Home Network Public Key Identifier, and the list of Protection Scheme Identifiers. The ME shall select the protection scheme from its supported schemes that has the highest priority in the list obtained from the USIM.

#### 5.6.6.2	Conformance requirement

CR 1	SUCI calculation procedure shall be performed by the ME if Service n°124 is available and Service n°125 is not available in EFUST

Reference:

-	TS 31.102 [19], clause 5.3.47.

CR 2	SUPI is available in EFSUPI_NAI if Service n°130 is available in EFUST

Reference:

-	TS 31.102 [19], clauses 4.4.11.10, 7.5.2 and Annex N.

CR 3	A subscriber identifier is in the form of a SUPI in NAI format

References:

-	TS 31.102 [19], clause 4.4.11.10.

CR 4	The SUPI may contain:

- a NSI, used for private networks as defined in TS 22.261 [36] or

- a GLI and an operator identifier of the 5GC operator, used for supporting FN-BRGs, as further described in TS 23.316 [56] or

- a GCI and an operator identifier of the 5GC operator, used for supporting FN-CRGs and 5G-CRG, as further described in TS 23.316 [56].

References:

-	TS 22.261 [36], clause 8.5;

-	TS 23.316 [56], clause 4.7.3, 4.7.4, 4.7.8 and 4.7.9.

CR 5	As part of the SUCI calculation performed by the ME, the ME performs the reading procedure for EFSUCI_Calc_Info.

Reference:

-	TS 31.102 [19], clause 4.11.8 and 5.3.47.

CR 6		The ME shall calculate the SUCI using the ECIES scheme profile B if highest priority of the protection schemes listed in the USIM is the ECIES scheme profile B.

Reference:

-	TS 31.102 [19], clauses 5.3.47;

-	TS 33.501 [24], clause Annex C.

#### 5.6.6.3	Test purpose

The purpose of this test is to verify that:

1)	that the READ EFSUCI_Calc_Info, EFRouting_Indicator and EFSUPI_NAI commands are performed correctly by the ME;

2)	the ME performs SUCI calculation procedure using the profile with the highest priority (i.e. ECIES scheme profile B and the Home Network Public Key).

#### 5.6.6.4	Method of test

##### 5.6.6.4.1	Initial conditions

The values of the 5G-NR UICC – non-IMSI SUPI Type as defined in clause 4.5.10 of the present document are used with the following exception:

EFSUPI_NAI (SUPI as Network Access Identifier)

Logically:			verylongusername1@3gpp.com

SUPI Type:	NSI

Username:	verylongusername1

Realm:		3gpp.com

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 80 | 1A | 76 | 65 | 72 | 79 | 6C | 6F | 6E | 67 | 75 | 73 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 65 | 72 | 6E | 61 | 6D | 65 | 31 | 40 | 33 | 67 | 70 | 70 |
|  | B25 | B26 | B27 | B28 |
|  | 2E | 63 | 6F | 6D |

The TT (NG-SS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):		244/083/000001

-	Access control:				unrestricted.

The NG-SS shall be configured with Home Network Private Key for profile B:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | F1 | AB | 10 | 74 | 47 | 7E | BC | C7 | F5 | 54 | EA | 1C |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 5F | C3 | 68 | B1 | 61 | 67 | 30 | 15 | 5E | 00 | 41 | AC |
|  | B25 | B26 | B27 | B28 | B29 | B30 | B31 | B32 |
|  | 44 | 7D | 63 | 01 | 97 | 5F | EC | DA |

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and runs an initial activation.

##### 5.6.6.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | READ EFUST, EFSUPI_NAI, EFRouting_Indicator and EFSUCI_Calc_Info | (Evaluation of service settings) Verification of the READ command and the EF contents | (CR 1) (CR 2) CR 5 | A.2/1 OR A.2/2 |
| 1a | ME | Perform SUCI calculation | The ME shall calculate the SUCI using the ECIES scheme profile B | (CR 6) |  |
| 2 | UE > TT | Send REGISTRATION REQUEST | The UE sends a REGISTRATION REQUEST with 5GS registration type IE as "initial registration" and 5GS mobile identity information element type "SUCI" | CR 1 CR 2 CR 3 CR 4 CR 6 |  |
| 3 | TT > UE | Send REGISTRATION ACCEPT | The TT sends a REGISTRATION ACCEPT with 5G-GUTI |  |  |
| 4 | UE > TT | Send REGISTRATION COMPLETE |  |  |  |

#### 5.6.5.5	Acceptance criteria

CR 1, CR 2, CR 3, CR 4 and CR 6 are implicitly verified in step 2). The conformance requirements are met if the 5GS mobile identity IE in the REGISTRATION REQUEST performed in step 2) includes the following values:

-	SUPI format:						1

-	NAI format for the SUCI:		type1.rid17.schid2.hnkey27.ecckey<ECC ephemeral public key>.cip<encryption of "verylongusername1">.mac<MAC tag value>@3gpp.com

-	SUPI Type:						1

-	Home Network Identifier:		3gpp.com

-	Routing indicator:				17

-	Protection scheme ID:			02

-	Home network public key ID:	27

-	Scheme output:					ECC ephemeral public key, encryption of "verylongusername1" and MAC tag value

CR 1 and CR 2 may be explicitly verified in step 1) if the READ commands on EFUST and EFSUPI_NAI are analysed using either of the methods A.2/1 or A.2/2. CRs for explicit contents verification of EFRouting_Indicator and EFSUCI_Calc_Info are not defined.

CR 5 is explicitly verified in step 1). The requirement is met if EFSUCI_Calc_Info is read (A.2/1 or A.2/2).

CR 6 may optionally be explicitly verified in step 1a) by any method tracing the ME behaviour.

Example with test data from 3GPP TS 33.501 [43] Annex C:

type1.rid17.schid2.hnkey27.ecckey03759BB22C563D9F4A6B3C1419E543FC2F39D6823F02A9D71162B39399218B244B.cipBE22D8B9F856A52ED381CD7EAF4CF2D525.mac3CDDC61A0A7882EB@3gpp.com

# 6	Security related Tests

## 6.1	PIN handling

### 6.1.1	Entry of PIN

#### 6.1.1.1	Definition and applicability

The PIN is a number used to authenticate the user to the UICC for security. Entry of the correct PIN allows PIN-protected data to be accessed over the UICC-Terminal interface.

#### 6.1.1.2	Conformance requirement

CR 1	Following insertion of the UICC and powering on the UE, the ME shall check the state of the PIN. If the PIN is enabled, the UE asks the user for PIN verification

CR 2	The VERIFY PIN function verifies the PIN presented by the ME to the UICC.

CR 3	Indication of a successful pin verification is presented to user.

Reference:

-	ETSI TS 102 221 [5], clauses 9 and 11.1.9;

-	TS 31.102 [19], clause 6;

-	TS 22.030 [44], clause 6.6.1.

#### 6.1.1.3	Test purpose

The purpose of this test is to verify that:

1)	the PIN verification procedure is performed by the UE correctly,

2)	the basic public MMI string is supported.

#### 6.1.1.4	Method of test

##### 6.1.1.4.1	Initial conditions

The values of the Default UICC as defined in clause 4.5.2 of the present document are used.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case with PIN enabled.

##### 6.1.1.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Run an initial activation |  |  |  |
| 2 | UE > USER | Ask for the PIN to be entered | The ME indicates to the User that a PIN needs to be entered | CR 1 |  |
| 3 | USER > UE | Enter the sequence "0000#" | The UE initiates the VERIFY PIN command on the UICC and gets a confirmation from the UICC. | CR 2 | A.2/1 OR A.2/2 |
| 4 | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be successful | CR 3 |  |

#### 6.1.1.5	Acceptance criteria

CR 1 is met if the ME asks the user for the PIN verification.

CR 2 can be explicitly verified at step 3) via supported options A.2/1 OR A.2/2, ensuring that the VERIFY PIN command is correctly sent via the ME to the UICC, with parameter P2 = "01".

CR 3 is verified on the ME. CR 3 is met if an indication is given to the user showing that this procedure was executed successfully, e.g. by displaying "OK".

### 6.1.2	Change of PIN

#### 6.1.2.1	Definition and applicability

The PIN may be changed by the user, by entering the old and new PIN. The length of the PIN is between 4 and 8 digits.

#### 6.1.2.2	Conformance requirement

CR1	The ME shall support the change of PIN procedure as defined in ETSI TS 102 221 [5], clause 11.1.10.

CR2	Indication of a successful pin change is presented to the user, and only the new pin shall be accepted for further PIN verification.

Reference:

-	ETSI TS 102 221 [8], clauses 9 and 11.1.10;

-	TS 31.102 [19], clause 6;

-	TS 22.030 [44], clause 6.6.2.

#### 6.1.2.3	Test purpose

The purpose of this test is to verify that:

1)	the PIN substitution procedure is performed correctly by the UE,

2)	the basic public MMI string is supported,

3)	the old PIN shall no longer be accepted,

4)	the new PIN shall be accepted.

#### 6.1.2.4	Method of test

##### 6.1.2.4.1	Initial conditions

The values of the Default UICC as defined in clause 4.5.2 of the present document are used.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case with PIN enabled.

##### 6.1.2.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Run an initial activation |  |  |  |
| 2 | USER > UE | Enter the sequence "0000#" when the UE is in the "PIN check" mode |  |  |  |
| 3 | USER > UE | After USIM initialisation is completed, enter "**04*0000*01234567*01234567#" or initiate an equivalent MMI dependent procedure to change the PIN from '0000' to '01234567' | The UE initiates the CHANGE PIN command on the UICC and gets a confirmation from the UICC. | CR 1 | A.2/1 OR A.2/2 |
| 4 | UE > USER | An indication is given to the user showing whether this procedure was successful |  | CR 2 |  |
| 5 | UE | Deactivate and re-activate |  |  |  |
| 6 | USER > UE | Enter the sequence "0000#" when the UE is in the "PIN check" mode | The UE verifies the entered PIN command on the UICC | CR 1 | A.2/1 OR A.2/2 |
| 7 | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be unsuccessful | CR 2 |  |
| 8 | UE | Deactivate and re-activate |  |  |  |
| 9 | USER > UE | Enter the sequence "1234567#" when the UE is in the "PIN check" mode | The UE verifies the entered PIN command on the UICC and gets a confirmation from the UICC | CR 1 | A.2/1 OR A.2/2 |
| 10 | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be successful | CR 2 |  |

#### 6.1.2.5	Acceptance criteria

CR 1 is explicitly verified at:

- step 3) by analysing the CHANGE PIN command;

- step 6) and step 9) by analysing the VERIFY PIN command;

sent via the ME to the UICC, with parameter P2 = "01" (via any supported option A.2/1 OR A.2/2).

CR 2 is verified at:

-	step 4) on the ME by ensuring that an indication of a successful execution of the command and the acceptance 		of the new PIN is given;

-	step 7) on the ME by ensuring that an indication of an unsuccessful execution of the command is given;

-	step 10) on the ME by ensuring that an indication of a successful execution of the command is given.

### 6.1.3	Unblock PIN

#### 6.1.3.1	Definition and applicability

After three consecutive wrong entries of the PIN, the PIN shall become blocked. The Unblock PIN command is used to unblock the PIN. This function may be performed whether or not the PIN is blocked.

#### 6.1.3.2	Conformance requirement

CR 1	The UE shall support the Unblock PIN command, as defined in ETSI TS 102 221 [8], clause 11.1.13.

CR 2	Indication of a successful and unsuccessful unblock PIN is presented to user

CR 3	Only the new pin shall be accepted for further PIN verification.

CR 4	Indication of a successful and unsuccessful pin verification is presented to the user and only the new pin shall be accepted for further PIN verification.

Reference:

-	ETSI TS 102 221 [8], clause 11.1.13;

-	TS 31.102 [19], clause 6;

-	TS 22.030 [44], clause 6.6.3.

#### 6.1.3.3	Test purpose

The purpose of this test is to verify that:

1)	the PIN unblocking procedure is performed correctly,

2)	the basic public MMI string is supported,

#### 6.1.3.4	Method of test

##### 6.1.3.4.1	Initial conditions

The values of the Default UICC as defined in clause 4.5.2 of the present document are used.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case with PIN enabled.

##### 6.1.3.4.2	Procedure

##### 6.1.3.4.2	Procedure

Sequence A:

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Run an initial activation |  |  |  |
| 2 | USER > UE | Enter the correct PIN | (If test 6.1.2 has been executed before this probably is "01234567#") |  |  |
| 3 | USER > UE | After USIM initialisation is completed, enter "**05*11111111*1234*1234#" | The UE initiates the UNBLOCK PIN command on the UICC and gets a confirmation from the UICC | CR 1 | A.2/1 OR A2/2 |
| 4 | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be successful | CR 2 |  |
| 5 | UE | Deactivate and re-activate |  |  |  |
| 6 | USER > UE | When the UE is in the "PIN check" mode, enter the new PIN: "1234#" | The UE verifies the entered PIN command on the UICC and gets a confirmation from the UICC | CR 3 | A.2/1 OR A2/2 |
| 7 | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be successful | CR 4 |  |
| 8 | UE | Deactivate and re-activate the UE |  |  |  |
| 9 | USER > UE | When the UE is in the "PIN check" mode, enter a wrong PIN three times | The UE verifies the entered PIN command on the UICC and gets a confirmation from the UICC | CR 3 | A.2/1 OR A2/2 |
| 10 | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be unsuccessful | CR 4 |  |
| 11 | USER > UE | Enter "**05*11111111*2468*2468#" | The UE verifies the entered UNBLOCK PIN command on the UICC and gets a confirmation from the UICC | CR 1 | A.2/1 OR A2/2 |
| 12 | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be successful | CR 2 |  |
| 13 | UE | Deactivate and re-activate the UE |  |  |  |
| 14 | USER > UE | When the UE is in the "PIN check" mode, enter the new PIN "2468#" | The UE verifies the entered PIN command on the UICC and gets a confirmation from the UICC | CR 3 | A.2/1 OR A2/2 |
| 15 | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be successful | CR 4 |  |

Sequence B:

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Run an initial activation |  |  |  |
| 2 | USER > UE | Enter the wrong PIN three times |  |  |  |
| 3 | USER > UE | The user shall initiate an MMI dependent procedure to unblock the PIN with unblock code '11111111' and a new PIN '2468' ("**05*11111111*1234*1234#") | The UE initiates the UNBLOCK PIN command on the UICC and gets a confirmation from the UICC | CR 1 | A.2/1 OR A2/2 |
| 4 | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be successful | CR 2 |  |
| 5 | UE | Deactivate and re-activate |  |  |  |
| 6 | USER > UE | When the UE is in the "PIN check" mode, enter the new PIN: "2468#" | The UE verifies the entered PIN command on the UICC and gets a confirmation from the UICC | CR 3 | A.2/1 OR A2/2 |
| 7 | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be successful | CR 4 |  |

#### 6.1.3.5	Acceptance criteria

Sequence A:

CR 1 is explicitly verified at step 3) and step 11) by analysing the UNBLOCK PIN command sent via the ME to the UICC, with parameter P2 = "01" (via any supported options A.2/1 OR A.2/2).

CR 2 is verified at step 4) and step 12) on the UE by ensuring UE shall give an indication following a successful execution of the command and new PIN is accepted.

CR 3 is explicitly verified at step 6), step 9) and step 14) by analysing the status word for both successful and unsuccessful PIN entry.

CR 4 is verified at step 7) and step 15) on the UE by ensuring UE shall give an indication of a successful execution of the command and PIN is accepted, at step 10) on UE by ensuring UE shall give an indication that execution of the command failed.

Sequence B:

CR 1 is explicitly verified at step 3) by analysing the UNBLOCK PIN command sent via the ME to the UICC, with parameter P2 = "01" (via any supported options A.2/1 OR A.2/2) after UE indicates that PIN has been blocked in step 2).

CR 2 is verified at step 4) on the ME by ensuring UE shall give an indication following a successful execution of the command and new PIN is accepted.

CR 3 is explicitly verified at step 6) by analysing the status word for successful PIN entry.

CR 4 is verified at step 7) the requirement is met if the UE gets an indication that the PIN is accepted after the successful entry of the correct PIN value.

### 6.1.4	Entry of PIN2

#### 6.1.4.1	Definition and applicability

The PIN2 is a number used to authenticate the user to the UICC for security. Entry of the correct PIN2 allows PIN2-protected data to be accessed over the UICC-Terminal interface.

#### 6.1.4.2	Conformance requirement

CR 1	Before allowing the access to PIN2 protected data, the ME shall ask the user for PIN2 verification. Only after presenting the PIN2, the user shall get access to these data.

CR 2	The VERIFY PIN function verifies the PIN2 presented by the ME to the UICC.

CR 3	Indication of a successful PIN2 verification is presented to the user.

Reference:

-	ETSI TS 102 221 [8], clauses 9 and 11.1.9;

-	TS 31.102 [19], clause 6;

-	TS 22.030 [44], clause 6.6.1.

#### 6.1.4.3	Test purpose

The purpose of this test is to verify that:

1)	the PIN2 verification procedure is performed by the UE correctly.

2)	the basic public MMI string is supported.

#### 6.1.4.4	Method of test

##### 6.1.4.4.1	Initial conditions

The values of the Default UICC as defined in clause 4.5.2 of the present document are used.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case with PIN enabled.

NOTE:	To perform the UPDATE FDN data (as described in the procedure below), the default FDN UICC may be used.

##### 6.1.4.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Run an initial activation |  |  |  |
| 2 | USER > UE | When the UE is in the "PIN check" mode, correct PIN shall be entered |  |  |  |
| 3 | USER > UE | The access to a PIN2 protected data field shall be performed (e.g. UPDATE FDN) |  |  |  |
| 4 | USER > UE | Enter the sequence "9999#" in "PIN2 check" mode | When the UE is in the "PIN2 check" mode, the sequence "9999#" shall be entered and the ME is verifying the command | CR 1 | A.2/1 OR A2/2 |
| 5 | UE > USER | An indication is given to the User showing whether this procedure was successful | This procedure shall be successful | CR 2 |  |

#### 6.1.4.5	Acceptance criteria

CR 1 is explicitly verified at step 4) via supported options A.2/1 OR A.2/2, ensuring that the VERIFY PIN command is correctly sent via the ME to the UICC, with parameter P2 = "81".

CR 2 is verified on the ME at step 5). CR 2 is met if an indication is given to the user showing that this procedure was executed successfully, e.g. by displaying "OK".

### 6.1.5	Change of PIN2

#### 6.1.5.1	Definition and applicability

The PIN2 may be changed by the user, by entering the old and new PIN2. The length of the PIN2 is between 4 and 8 digits.

#### 6.1.5.2	Conformance requirement

CR 1	The ME shall support the change of PIN2 procedure as defined in ETSI TS 102 221 [5], clause 11.1.10.

CR 2	Indication of a successful pin change is presented to user, and only the new pin shall be accepted for further PIN verification.

Reference:

-	ETSI TS 102 221 [8], clauses 9 and 11.1.10;

-	TS 31.102 [19], clause 6;

-	TS 22.030 [44], clause 6.6.2.

#### 6.1.5.3	Test purpose

The purpose of this test is to verify that:

1)	the PIN2 substitution procedure is performed correctly by the UE.

2)	the basic public MMI string is supported.

3)	the old PIN shall no longer be accepted.

4)	the new PIN shall be accepted.

#### 6.1.5.4	Method of test

##### 6.1.5.4.1	Initial conditions

The values of the Default UICC as defined in clause 4.5.2 of the present document are used.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case with PIN enabled and runs an initial activation with the correct PIN entered.

NOTE:	To perform the UPDATE FDN data (as described in the procedure below), the default FDN UICC may be used.

##### 6.1.5.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | USER > UE | After USIM initialisation enter "**042*9999*12345678*12345678#" or initiate an equivalent MMI dependent procedure to change the PIN2 from '9999' to '12345678' | The UE initiates the CHANGE PIN command for PIN 2 on the UICC and gets a confirmation from the UICC. | CR 1 | A.2/1 OR A.2/2 |
| 2 | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be successful | CR 2 |  |
| 3 | UE | Deactivate and re-activate the UE |  |  |  |
| 4 | USER > UE | Enter the correct PIN |  |  |  |
| 5 | USER > UE | Access a PIN2 protected data field (e.g. perform an UPDATE FDN) |  |  |  |
| 6 | USER > UE | When the UE is in the "PIN2 check" mode, enter "9999#" | An incorrect PIN2 "9999#" is entered and cannot be verified | CR 1 | A.2/1 OR A2/2 |
| 7 | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be unsuccessful | CR 2 |  |
| 8 | UE | Deactivate and re-activate the UE |  |  |  |
| 9 | USER > UE | Enter the correct PIN |  |  |  |
| 10 | USER > UE | Access a PIN2 protected data field (e.g. perform an UPDATE FDN) |  |  |  |
| 11 | USER > UE | When the UE is in the "PIN2 check" mode, enter "12345678#" | The correct PIN2 is entered and shall be verified | CR 1 | A.2/1 OR A2/2 |
| 12 | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be successful | CR 2 |  |

#### 6.1.5.5	Acceptance criteria

CR 1 is explicitly verified at:

-	step 1) by analysing the CHANGE PIN command;

-	step 6) and step 11) by analysing the VERIFY PIN sent via the ME to the UICC, with parameter P2 = "81"
		 (via any supported options A.2/1 OR A.2/2).

CR 2 is verified (i) at:

-	step 2). CR 2 is met if an indication is given to the user showing that this procedure was executed successfully,
		and new PIN is accepted, e.g. by displaying "OK";

-	step 7). CR 2 is met if an indication is given to the user showing that the command failed;

-	step 12). CR 2 is met if an indication is given to the user showing that the command was executed successfully.

### 6.1.6	Unblock PIN2

#### 6.1.6.1	Definition and applicability

After three consecutive wrong entries of the PIN2, the PIN2 shall become blocked. The Unblock PIN2 command is used to unblock the PIN2. This function may be performed whether or not the PIN2 is blocked.

#### 6.1.6.2	Conformance requirement

CR 1	The UE shall support the Unblock PIN2 command, as defined in ETSI TS 102 221 [5], clause 11.1.13.

CR 2	Indication of a successful unblock PIN2 is presented to user

CR 3	Only the new PIN2 shall be accepted for further PIN2 verification.

CR 4	Indication of a successful and unsuccessful PIN2 verification is presented to user and only the new PIN2 shall be accepted for further PIN2 verification.

Reference:

-	ETSI TS 102 221 [8], clauses 9 and 11.1.13;

-	TS 31.102 [19], clause 6;

-	TS 22.030 [44], clause 6.6.3.

#### 6.1.6.3	Test purpose

The purpose of this test is to verify that:

1)	the PIN2 unblocking procedure is performed correctly.

2)	the basic public MMI string is supported.

#### 6.1.6.4	Method of test

##### 6.1.6.4.1	Initial conditions

The values of the Default UICC as defined in clause 4.5.2 of the present document are used.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case with PIN/PIN2 enabled.

NOTE:	To perform the UPDATE FDN data (as described in the procedure below), the default FDN UICC may be used.

##### 6.1.6.4.2	Procedure

Sequence A:

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | USER > UE | Run an initial activation of the UE and enter the correct PIN |  |  |  |
| 2 | USER > UE | After USIM initialisation enter "**052*22222222*1234*1234#" | The UE sends the UNBLOCK PIN command with parameter P2 = "81" and gets a confirmation from the UICC | CR 1 | A.2/1 OR A.2/2 |
| 3 | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be successful | CR 2 |  |
| 4 | UE | Deactivate and re-activate the UE |  |  |  |
| 5 | USER > UE | Access a PIN2 protected data field (e.g. UPDATE FDN) |  |  |  |
| 6 | USER > UE | Enter the new PIN2: "1234#" | The correct new PIN2 is entered The UE verifies the entered PIN2 command on the UICC and gets a confirmation from the UICC | CR 3 | A.2/1 OR A.2/2 |
| 7 | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be successful | CR 2 CR 3 CR 4 |  |
| 8 | UE | Deactivate and re-activate the UE |  |  |  |
| 9 | USER | The access to a PIN2 protected data field shall be performed (e.g. UPDATE FDN) |  |  |  |
| 10 | USER > UE | Enter a wrong PIN2 three times. | The UE verifies the entered PIN command on the UICC and gets a confirmation from the UICC | CR 3 | A.2/1 OR A.2/2 |
| 11 | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be unsuccessful | CR 4 |  |
| 12 | USER > UE | Enter "**052*22222222*3579*3579#". | The UE sends the UNBLOCK PIN command with parameter P2 = "81" to the UICC and gets a confirmation from the UICC | CR 1 | A.2/1 OR A.2/2 |
| 13 | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be successful | CR 2 |  |
| 14 | UE | The UE is powered off and on and the correct PIN is entered |  |  |  |
| 15 | USER > UE | The access to a PIN2 protected data field shall be performed (e.g. UPDATE FDN) |  |  |  |
| 16 | USER > UE | Enter the new PIN2: "3579#" | The correct new PIN2 is entered The UE verifies the entered PIN2 command on the UICC and gets a confirmation from the UICC | CR 3 | A.2/1 OR A.2/2 |
| 17 | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be successful | CR 4 |  |

Sequence B:

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Run initial activation of the UE and enter the correct PIN |  |  |  |
| 2 | USER > UE | Access a PIN2 protected data field (e.g. UPDATE FDN) |  |  |  |
| 3 | USER > UE | Enter a wrong PIN2 three times. | The UE verifies the entered PIN command on the UICC and gets a confirmation from the UICC |  |  |
| 4 | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be unsuccessful | CR 4 |  |
| 5 | USER > UE | The user shall initiate an MMI dependent procedure to unblock the PIN2 with unblock code '22222222' and a new PIN '3579' | The UE sends the UNBLOCK PIN command with correct parameters to the UICC and gets a confirmation from the UICC | CR 1 | A.2/1 OR A.2/2 |
| 6 | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be successful | CR 2 |  |
| 7 | UE | The UE is powered off and on and the correct PIN is entered. |  |  |  |
| 8 | USER | The access to a PIN2 protected data field shall be performed (e.g. UPDATE FDN) |  |  |  |
| 9 | USER > UE | Enter the new PIN2: "3579#" | The UE sends the VERIFY PIN command with correct parameters to the UICC and gets a confirmation from the UICC | CR 3 | A.2/1 OR A.2/2 |
| 10 | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be successful | CR 4 |  |

#### 6.1.6.5	Acceptance criteria

Sequence A:

CR 1	is explicitly verified at step 2) and step 12) by analysing the UNBLOCK PIN command sent via ME to the UICC, with parameter P2 = "81" (via any supported options A.2/1 OR A.2/2).

CR 2	is verified at step 3), step 7) and step 15) on the UE by ensuring UE shall give an indication of a successful execution of the command and new PIN2 is accepted.

CR 3	is implicitly verified at step 7) if the successful execution is indicated.

CR 3	can be explicitly verified in step 6), step 10) and step 16) for both successful and unsuccessful PIN2 entry.

CR 4	is verified at step 7) and step 17) on the UE by ensuring UE shall give an indication of a successful execution of the command and PIN2 is accepted, at step 11) on UE by ensuring UE shall give an indication that execution of the command failed.

Sequence B:

CR 1	is explicitly verified at step 5) by analysing the UNBLOCK PIN command sent via ME to the UICC, with parameter P2 = "81" (via any supported options A.2/1 OR A.2/2) after UE indicates that PIN has been blocked in step 4).

CR 2	is verified at step 6) on the UE by ensuring UE shall give an indication following a successful execution of the command and new PIN2 is accepted.

CR 3	is explicitly verified at step 8) for a successful PIN2 entry.

CR 4	is verified at step 4) on the UE by ensuring UE shall give an indication following an unsuccessful execution and at step 10) providing an information following a successful execution of the command that PIN2 is accepted.

### 6.1.7	Replacement of PIN

#### 6.1.7.1	Definition and applicability

The Universal PIN may be used to replace a PIN used to authenticate the user to the UICC for security. In this case entry of the correct Universal PIN allows PIN-protected data to be accessed over the UICC-Terminal interface. ETSI TS 102 221 [8] defines the range of "01" to "08" as key reference of the PIN on a multi-verification capable UICC.

#### 6.1.7.2	Conformance requirement

CR 1	The UE shall support the usage of the Universal PIN as replacement PIN and the replacement procedure as defined in ETSI TS 102 221 [8], clause 11.1.11.

CR 2	The UE shall support the procedure to disable the replacement defined in ETSI TS 102 221 [5], clause 11.1.12.

CR 3	Only the new pin shall be accepted for further PIN verification.

CR 4	Indication of a successful and unsuccessful pin verification is presented to the user and only the new pin shall be accepted for further PIN verification

Reference:

-	ETSI TS 102 221 [8], clauses 9, 11.1.11 and 11.1.12;

-	TS 31.102 [19], clause 6.

#### 6.1.7.3	Test purpose

The purpose of this test is to verify that:

1)	the PIN replacement is supported by the UE correctly.

2)	the PIN replacement procedure is performed by the UE correctly.

3)	the procedure to disable the PIN replacement is performed by the UE correctly.

#### 6.1.7.4	Method of test

##### 6.1.7.4.1	Initial conditions

The values of the Default UICC as defined in clause 4.5.2 of the present document are used.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case with PIN enabled.

##### 6.1.7.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Run initial activation. |  |  |  |
| 2 | USER > UE | When the UE is in the "PIN check" mode, the sequence "2468#" shall be entered. |  |  |  |
|  | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be successful |  |  |
| 3 | USER > UE | The user shall initiate an MMI dependent procedure to replace the PIN by the Universal PIN. |  |  |  |
|  | UE > UICC | DISABLE PIN | This is verifiable only if A2/x is supported. | CR 1 |  |
|  | USIM > UE | Check Status word |  |  |  |
| 4 | UE > USER | The UE is powered off and on. |  |  |  |
| 5 | USER > UE | When the UE is in the "PIN check" mode, the sequence "2468#" shall be entered. |  |  |  |
|  | UE > UICC | VERIFY PIN | This is verifiable only if A2/x is supported. | CR 3 | A.2/1 OR A2/2 |
|  | USIM > UE | Check Status word |  |  |  |
|  | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be unsuccessful | CR 4 |  |
| 6 |  | The correct Universal PIN is entered. |  |  |  |
|  | UE > UICC | VERIFY PIN | This is verifiable only if A2/x is supported. | CR 3 | A.2/1 OR A2/2 |
|  | USIM > UE | Check Status word |  |  |  |
|  | UE | An indication is given to the user showing whether this procedure was successful | This procedure shall be successful | CR 4 |  |
| 7 |  | Initiate an MMI dependent procedure to disable the replacement of the PIN by the Universal PIN |  |  |  |
|  | UE > UICC | ENABLE PIN | This is verifiable only if A2/x is supported. | CR 2 | A.2/1 OR A2/2 |
|  | USIM > UE | Check Status word |  |  |  |
| 8 | UE > USER | The UE is powered off and on. |  |  |  |
| 9 | USER > UE | When the UE is in the "PIN check" mode, the sequence "2839#" shall be entered. |  |  |  |
|  | UE > UICC | VERIFY PIN | This is verifiable only if A2/x is supported. | CR 3 | A.2/1 OR A2/2 |
|  | USIM > UE | Check Status word |  |  |  |
|  | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be unsuccessful | CR 4 |  |
| 10 | USER > UE | When the UE is in the "PIN check" mode correct PIN is entered. |  |  |  |
|  | UE > UICC | VERIFY PIN | This is verifiable only if A2/x is supported. | CR 3 | A.2/1 OR A2/2 |
|  | USIM > UE | Check Status word |  |  |  |
|  | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be successful | CR 4 |  |

#### 6.1.7.5	Acceptance criteria

CR 1 is explicitly verified at step 3 by analysing the DISABLE PIN command sent via the ME to the UICC, with parameter P1 = "91" and P2 = "01" (via any supported options A.2/1 OR A2/2).

CR 2 is explicitly verified at step 7) by analysing the ENABLE PIN command sent via the ME to the UICC, with parameter P2 = "01" (via any supported options A.2/1 OR A2/2)

CR 3 is explicitly verified at (i) step 5) by analysing the status word indicating that the PIN has not been accepted for the VERIFY PIN command sent by the ME to the UICC, with parameter P2 = "11", (ii) step 6) by analysing the status word indicating that the PIN has been accepted for the VERIFY PIN command sent by the ME to the UICC, with parameter P2 = "11", (iii) step 9) by analysing the status word indicating that the PIN has not been accepted for the VERIFY PIN command sent by the ME to the UICC, with parameter P2 = "01", (ii) step 10) by analysing the status word indicating that the PIN has been accepted for the VERIFY PIN command sent by the terminal to the UICC, with parameter P2 = "01".

CR 4 is verified (i) at step 5) and 9) on the UE by ensuring UE shall give an indication of an unsuccessful execution of the command and PIN is not accepted, (ii) at step 6) and 10) on the UE by ensuring UE shall give an indication of a successful execution of the command and PIN is accepted.

### 6.1.8	Change of Universal PIN

#### 6.1.8.1	Definition and applicability

The Universal PIN may be changed by the user, by entering the old and new Universal PIN. The length of the Universal PIN is between 4 and 8 digits.

#### 6.1.8.2	Conformance requirement

CR 1	The UE shall support the change of PIN procedure as defined in ETSI TS 102 221 [8], clause 11.1.10.

CR 2	Only the new pin shall be accepted for further PIN verification.

CR 3	Indication of a successful and unsuccessful pin verification is presented to the user and only the new pin shall be accepted for further PIN verification

Reference:

-	ETSI TS 102 221 [8], clauses 9 and 11.1.10;

-	TS 31.102 [19], clause 6.

#### 6.1.8.3	Test purpose

The purpose of this test is to verify that the PIN substitution procedure is performed correctly by the ME.

#### 6.1.8.4	Method of test

##### 6.1.8.4.1	Initial conditions

The values of the Default UICC as defined in clause 4.5.2 of the present document are used with the following exception:

-	The Universal PIN is used as a replacement of the PIN.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case with PIN enabled and runs an initial activation with the correct Universal PIN entered.

##### 6.1.8.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | USER > UE | Initiate an MMI dependent procedure to change the Universal PIN to "01234567". | The UE sends the CHANGE PIN command with parameter P2 = "81" |  |  |
|  | UE > UICC | CHANGE PIN | This is verifiable only if A.2/x is supported. | CR 1 | A.2/1 OR A2/2 |
|  | USIM> UE | Check Status word |  |  |  |
| 2 | UE | The UE is powered off and on. |  |  |  |
| 3 | USER > UE | When the UE is in the "PIN check" mode, enter the new PIN: "2839#" |  |  |  |
|  | UE > UICC | VERIFY PIN | This is verifiable only if A.2/x is supported. | CR 2 | A.2/1 OR A.2/2 |
|  | USIM > UE | Check Status word |  |  |  |
|  | UE | An indication is given to the user showing whether this procedure was successful | This procedure shall be unsuccessful | CR 3 |  |
| 4 | UE | The UE is powered off and on. |  |  |  |
| 5 | USER > UE | When the UE is in the "PIN check" mode, enter the new PIN: "01234567#" |  |  |  |
|  | UE > UICC | VERIFY PIN | This is verifiable only if A.2/x is supported. | CR 2 | A.2/1 OR A.2/2 |
|  | USIM > UE | Check Status word |  |  |  |
|  | UE | An indication is given to the user showing whether this procedure was successful | This procedure shall be successful | CR 3 |  |

#### 6.1.8.5	Acceptance criteria

CR 1 is explicitly verified at step 1) by analysing both the CHANGE PIN command sent via the ME to the UICC, with parameter P2 = "11" (via any supported options A.2/1 OR A.2/2) and the corresponding status word indicating that the new (Universal) PIN is accepted.

CR 2 is explicitly verified at (i) step 3) by analysing the status word indicating that the entered (Universal) PIN has not been accepted for the VERIFY PIN command sent by the ME to the UICC, and at (ii) step 5) by analysing the status word indicating that the entered (Universal) PIN has been accepted for the VERIFY PIN command sent by the ME to the UICC.

CR 3 is verified at (i) step 3) on the UE by ensuring UE shall give an indication following an unsuccessful execution of the command and at (ii) step 5) on the UE by ensuring UE shall give an indication following a successful execution of the command.

### 6.1.9	Unblock Universal PIN

#### 6.1.9.1	Definition and applicability

After three consecutive wrong entries of the PIN, the PIN shall become blocked. The Unblock PIN command is used to unblock the PIN. This function may be performed whether or not the PIN is blocked.

#### 6.1.9.2	Conformance requirement

CR 1	The UE shall support the Unblock PIN command, as defined in ETSI TS 102 221 [8], clause 11.1.13.

CR 2	Only the new pin shall be accepted for further PIN verification.

CR 3	Indication of a successful and unsuccessful pin verification is presented to the user and only the new pin shall be accepted for further PIN verification

Reference:

-	ETSI TS 102 221 [8], clause 11.1.13;

-	TS 31.102 [19], clause 6.

#### 6.1.9.3	Test purpose

To verify that the PIN unblocking procedure is performed correctly.

#### 6.1.9.4	Method of test

##### 6.1.9.4.1	Initial conditions

The values of the Default UICC as defined in clause 4.5.2 of the present document are used.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case with PIN enabled.

##### 6.1.9.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Run initial activation of the UE and enter the correct PIN |  |  |  |
| 2 | USER > UE | The user shall initiate an MMI dependent procedure to unblock the Universal PIN and set the new Universal PIN value to "1234" |  |  |  |
|  | UE > UICC | UNBLOCK PIN | This is verifiable only if A.2/x is supported. | CR 1 | A.2/1 OR A.2/2 |
|  | USIM > UE | Check Status word |  |  |  |
| 3 | UE | The UE is powered off and on. |  |  |  |
| 4 | USER > UE | Enter the new PIN: "1234#" |  |  |  |
|  | UE > UICC | VERIFY PIN | This is verifiable only if A.2/x is supported. | CR 2 | A.2/1 OR A.2/2 |
|  | USIM > UE | Check Status word |  |  |  |
|  | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be successful | CR 3 |  |
| 5 | UE | The UE is powered off and on. |  |  |  |
| 6 | USER > UE | Enter a wrong PIN three times. |  |  |  |
|  | UE > UICC | VERIFY PIN | This is repeated 3 times, and is verifiable only if A.2/x is supported. | CR 2 | A.2/1 OR A.2/2 |
|  | USIM> UE | Check Status word |  |  |  |
|  | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be unsuccessful | CR 3 |  |
| 7 | USER > UE | The user shall initiate an MMI dependent procedure to unblock the Universal PIN and set the new Universal PIN value to "2839". |  |  |  |
|  | UE > UICC | UNBLOCK PIN | This is verifiable only if A2/x is supported. | CR 1 | A.2/1 OR A.2/2 |
|  | USIM> UE | Check Status word |  |  |  |
| 8 | UE | The UE is powered off and on. |  |  |  |
| 9 | USER > UE | Enter the new PIN: "2839#" |  |  |  |
|  | UE > UICC | VERIFY PIN | This is verifiable only if A.2/x is supported. | CR 2 | A.2/1 OR A.2/2 |
|  | USIM > UE | Check Status word |  |  |  |
|  | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be successful | CR 3 |  |

#### 6.1.9.5	Acceptance criteria

CR 1 is explicitly verified at step 2) and step 7) by analysing both the UNBLOCK PIN command sent via the ME to the UICC, with parameter P2 = "11" (via any supported options A.2/1 OR A.2/2) and the corresponding status word indicating that the new (Universal) PIN is accepted.

CR 2 is explicitly verified at (i) step 4) and step 9) by analysing the status word indicating that the entered (Universal) PIN has been accepted for the VERIFY PIN command sent by the ME to the UICC, and at (ii) step 6 by analysing each status word(s) indicating that the entered (Universal) PIN has not been accepted for 3 consecutive VERIFY PIN command sent by the ME to the UICC and after the third entry the (Universal) PIN has been blocked.

CR 3 is verified (i) at step 4) on UE by either ensuring UE shall give an indication of a successful execution of the command and new (Universal) PIN is accepted, (ii) at step 6 on UE by ensuring UE shall give an indication that execution of the command failed and  (Universal) PIN is blocked, (iii) at step 9) on UE by ensuring UE shall give an indication of a successful execution of the command and new (Universal) PIN is accepted.

### 6.1.10	Entry of PIN on multi-verification capable UICCs

#### 6.1.10.1	Definition and applicability

The PIN is a number used to authenticate the user to the UICC for security. Entry of the correct PIN allows PIN-protected data to be accessed over the UICC-Terminal interface. ETSI TS 102 221 [5] defines the range of "01" to "08" as key reference of the PIN on a multi-verification capable UICC.

#### 6.1.10.2	Conformance requirement

CR 1	Following insertion of the UICC and powering on the UE, the ME shall check the state of the PIN. If the PIN is enabled, the UE asks the user for PIN verification.

CR 2	The VERIFY PIN function verifies the PIN presented by the ME to the UICC.

CR 3	Indication of a successful PIN verification is presented to user.

Reference:

-	ETSI TS 102 221 [8], clauses 9 and 11.1.9;

-	TS 31.102 [19], clause 6;

-	TS 22.030 [44], clause 6.6.1.

6.1.10.3	Test purpose

The purpose of this test is to verify that:

1)	the PIN verification procedure is performed by the UE correctly.

2)	the basic public MMI string is supported.

3)	the UE supports key references in the range of "01" to "08" as PIN.

#### 6.1.10.4	Method of test

##### 6.1.10.4.1	Initial conditions

The values of the Default UICC as defined in clause 4.5.2 of the present document are used with the following exceptions:

The UICC shall be configured to use "07" as the key reference of the PIN and "87" as key reference of the PIN2 with the following values:

PIN

Key reference:	07

Logically:

8642

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 38 | 36 | 34 | 32 | FF | FF | FF | FF |

Unblock PIN

Key reference:	07

Logically:

64534231

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 36 | 34 | 35 | 33 | 34 | 32 | 33 | 31 |

PIN2

Key reference:	87

Logically:

9753

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 39 | 37 | 35 | 33 | FF | FF | FF | FF |

Unblock PIN2

Key reference:	87

Logically:

57687980

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 35 | 37 | 36 | 38 | 37 | 39 | 38 | 30 |

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case with PIN enabled.

##### 6.1.10.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Run initial activation. |  |  |  |
| 2 | USER > UE | When the UE is in the "PIN check" mode, the sequence "8642#" shall be entered |  | CR 1 |  |
|  | UE > UICC | VERIFY PIN | This is verifiable only if A.2/x is supported. | CR 2 | A.2/1 OR A.2/2 |
|  | USIM > UE | Check Status word |  |  |  |
|  | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be successful | CR 3 |  |

#### 6.1.10.5	Acceptance criteria

CR 1 is met if the UE asks the user for the PIN verification.

CR 2 can be explicitly verified at step 2) via supported options A.2/1 OR A.2/2, ensuring that the VERIFY PIN command is correctly sent via the ME to the UICC, with parameter P2 = "07".

CR 3 is verified on the UE. CR 3 is met if an indication is given to the user showing that this procedure was executed successfully, e.g. by displaying: "OK".

### 6.1.11	Change of PIN on multi-verification capable UICCs

#### 6.1.11.1	Definition and applicability

The PIN may be changed by the user, by entering the old and new PIN. The length of the PIN is between 4 and 8 digits. ETSI TS 102 221 [8] defines the range of "01" to "08" as key reference of the PIN on a multi-verification capable UICC.

#### 6.1.11.2	Conformance requirement

CR 1	The UE shall support the change of PIN procedure as defined in ETSI TS 102 221 [5], clause 11.1.10.

CR 2	Indication of a successful pin change is presented to user, and only the new pin shall be accepted for further PIN verification.

Reference:

-	ETSI TS 102 221 [8], clauses 9 and 11.1.10;

-	TS 31.102 [19], clause 6;

-	TS 22.030 [44], clause 6.6.2.

#### 6.1.11.3	Test purpose

The purpose of this test is to verify that:

1)	the PIN substitution procedure is performed correctly by the UE.

2)	the basic public MMI string is supported.

3)	the UE supports key references in the range of "01" to "08" as PIN.

#### 6.1.11.4	Method of test

##### 6.1.11.4.1	Initial conditions

The values of the Default UICC as defined in clause 4.5.2 of the present document are used  with the following exceptions:

The UICC shall be configured to use "07" as the reference of the PIN and "87" as reference of the PIN2 with the following values:

PIN

Key reference:	07

Logically:

8642

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 38 | 36 | 34 | 32 | FF | FF | FF | FF |

Unblock PIN

Key reference:	07

Logically:

64534231

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 36 | 34 | 35 | 33 | 34 | 32 | 33 | 31 |

PIN2

Key reference:	87

Logically:

9753

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 39 | 37 | 35 | 33 | FF | FF | FF | FF |

Unblock PIN2

Key reference:	87

Logically:

57687980

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 35 | 37 | 36 | 38 | 37 | 39 | 38 | 30 |

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case with PIN enabled and runs an initial activation with the correct PIN entered.

##### 6.1.11.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | USER > UE | After USIM initialisation is completed, enter "**04*8642*01234567*01234567#" or initiate an equivalent MMI dependent procedure to change the PIN from '8642' to '01234567' |  |  |  |
|  | UE > UICC | CHANGE PIN | This is verifiable only if A2/x is supported. | CR 1 | A.2/1 OR A.2/2 |
|  | UICC > UE | Check Status word |  |  |  |
|  | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure is successful | CR 2 |  |
| 2 | UE | The UE is powered off and on. |  |  |  |
| 3 | USER > UE | When the UE is in the "PIN check" mode, the sequence "8642#" shall be entered. |  |  |  |
|  | UE > UICC | VERIFY PIN | This is verifiable only if A2/x is supported. | CR 1 | A.2/1 OR A.2/2 |
|  | USIM > UE | Check Status word |  |  |  |
|  | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure is unsuccessful | CR 2 |  |
| 4 | UE | The UE is powered off and on. |  |  |  |
| 5 | USER > UE | When the UE is in the "PIN check" mode, the sequence "01234567#" shall be entered. |  |  |  |
|  | UE > UICC | VERIFY PIN | This is verifiable only if A2/x is supported. | CR 1 | A.2/1 OR A.2/2 |
|  | USIM > UE | Check Status word |  |  |  |
|  | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure is successful | CR 2 |  |

#### 6.1.11.5	Acceptance criteria

CR 1 is explicitly verified (i) at step 1) by analysing the CHANGE PIN command (ii) at step 3) and step 5) by analysing the VERIFY PIN sent via the ME to the UICC, with parameter P2 = "07" (via any supported options A.2/1 OR A2/2).

CR 2 is verified (i) at step 1) on UE by either ensuring UE shall give an indication of a successful execution of the command and new PIN is accepted, (ii) at step 3) on UE by ensuring UE shall give an indication that execution of the command failed, (iii) at step 5) on UE by ensuring UE shall give an indication of a successful execution of the command.

### 6.1.12	Unblock PIN on multi-verification capable UICCs

#### 6.1.12.1	Definition and applicability

After three consecutive wrong entries of the PIN, the PIN shall become blocked. The Unblock PIN command is used to unblock the PIN. This function may be performed whether or not the PIN is blocked. ETSI TS 102 221 [8] defines the range of "01" to "08" as key reference of the PIN on a multi-verification capable UICC.

#### 6.1.12.2	Conformance requirement

CR 1	The UE shall support the Unblock PIN command, as defined in ETSI TS 102 221 [8], clause 11.1.13.

CR 2	Indication of a successful and unsuccessful unblock PIN is presented to user.

CR 3	Only the new pin shall be accepted for further PIN verification.

CR 4	Indication of a successful and unsuccessful pin verification is presented to the user and only the new pin shall be accepted for further PIN verification.

Reference:

-	ETSI TS 102 221 [8], clauses 9 and 11.1.13;

-	TS 31.102 [19], clause 6;

-	TS 22.030 [44], clause 6.6.3.

#### 6.1.12.3	Test purpose

The purpose of this test is to verify that:

1)	the PIN unblocking procedure is performed correctly.

2)	the basic public MMI string is supported.

3)	the ME supports key references in the range of "01" to "08" as PIN.

#### 6.1.12.4	Method of test

##### 6.1.12.4.1	Initial conditions

The values of the Default UICC as defined in clause 4.5.2 of the present document are used with the following exceptions:

The UICC shall be configured to use "07" as the reference of the PIN and "87" as reference of the PIN2 with the following values:

PIN

Key reference:	07

Logically:			8642

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 38 | 36 | 34 | 32 | FF | FF | FF | FF |

Unblock PIN

Key reference:	07

Logically:			64534231

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 36 | 34 | 35 | 33 | 34 | 32 | 33 | 31 |

PIN2

Key reference:	87

Logically:			9753

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 39 | 37 | 35 | 33 | FF | FF | FF | FF |

Unblock PIN2

Key reference:	87

Logically:			57687980

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 35 | 37 | 36 | 38 | 37 | 39 | 38 | 30 |

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case with PIN enabled.

##### 6.1.12.4.2	Procedure

Sequence A:

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | The ME is powered on and the correct PIN is entered. |  |  |  |
| 2 | USER > UE | After USIM initialisation is completed, enter "**05*64534231*1234*1234#" |  |  |  |
|  | UE > UICC | UNBLOCK PIN | This is verifiable only if A.2/x is supported. | CR 1 | A.2/1 OR A.2/2 |
|  | USIM > UE | Check Status word |  |  |  |
|  | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be successful | CR 2 |  |
| 3 | UE | The ME is powered off and on. |  |  |  |
| 4 | USER > UE | When the UE is in the "PIN check" mode, enter the new PIN: "1234#" |  |  |  |
|  | UE > UICC | VERIFY PIN | This is verifiable only if A.2/x is supported. | CR 3 | A.2/1 OR A.2/2 |
|  | USIM > UE | Check Status word |  |  |  |
|  | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be successful | CR 4 |  |
| 5 | UE | The ME is powered off and on. |  |  |  |
| 6 | USER > UE | Enter a wrong PIN three times. |  |  |  |
|  | UE > UICC | VERIFY PIN | This is repeated 3x, and is verifiable only if A.2/x is supported. | CR 3 | A.2/1 OR A2/2 |
|  | USIM > UE | Check Status word |  |  |  |
|  | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be unsuccessful | CR 4 |  |
| 7 | USER > UE | Enter "**05*64534231*8642*8642#". |  |  |  |
|  | UE > UICC | UNBLOCK PIN | This is verifiable only if A.2/x is supported. | CR 1 | A.2/1 OR A.2/2 |
|  | USIM > UE | Check Status word |  |  |  |
|  | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be successful | CR 2 |  |
| 8 | USER > UE | When the UE is in the "PIN check" mode, enter the new PIN: "8642#" |  |  |  |
|  | UE > UICC | VERIFY PIN | This is verifiable only if A.2/x is supported. | CR3 | A.2/1 OR A.2/2 |
|  | USIM > UE | Check Status word |  |  |  |
|  | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be successful | CR4 |  |

Sequence B:

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | The ME is powered on. |  |  |  |
| 2 | USER > UE | Enter a wrong PIN three times. |  |  |  |
| 3 | USER > UE | The user shall initiate a MMI dependent procedure to unblock the PIN with unblock code '64534231' and a new PIN '8642' ("**05*64534231*8642*8642#") . |  |  |  |
|  | UE > UICC | UNBLOCK PIN | This is verifiable only if A.2/x is supported. | CR 1 | A.2/1 OR A.2/2 |
|  | USIM > UE | Check Status word |  |  |  |
|  | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be successful | CR 2 |  |
| 4 | UE | The ME is powered off and on. |  |  |  |
| 5 | USER > UE | When the UE is in the "PIN check" mode, enter the new PIN: "8642#" |  |  |  |
|  | UE > UICC | VERIFY PIN | This is verifiable only if A.2/x is supported. | CR 3 | A.2/1 OR A2/2 |
|  | USIM > UE | Check Status word |  |  |  |
|  | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be successful | CR 4 |  |

#### 6.1.12.5	Acceptance criteria

Sequence A:

CR 1	is explicitly verified at step 2) and step 7) by analysing the UNBLOCK PIN command sent via the ME to the UICC, with parameter P2 = "07" (via any supported options A.2/1 OR A.2/2).

CR 2	is verified at step 2) and step 7) on the UE by ensuring UE shall give an indication following a successful execution of the command and new PIN is accepted.

CR 3	is explicitly verified at step 4), step 6) and step 8) by analysing the status word for both successful and unsuccessful PIN entry.

CR 4	is verified at (i) step 4) and step 8) on the UE by ensuring UE shall give an indication of a successful execution of the command and PIN is accepted, (ii) at step) on ME by ensuring UE shall give an indication that execution of the command failed.

Sequence B:

CR 1	is explicitly verified at step 3) by analysing the UNBLOCK PIN command sent via the ME to the UICC, with parameter P2 = "07" (via any supported options A.2/1 OR A.2/2) after UE indicates that PIN has been blocked in step 2).

CR 2	is verified at step 3) on the UE by ensuring UE shall give an indication following a successful execution of the command and new PIN is accepted.

CR 3	is explicitly verified at step 5) by analysing the Status word for successful PIN entry.

CR 4 is verified at step 5) on the UE by ensuring UE shall give an indication following a successful execution of the command when a PIN is accepted.

### 6.1.13	Entry of PIN2 on multi-verification capable UICCs

#### 6.1.13.1	Definition and applicability

The PIN2 is a number used to authenticate the user to the UICC for security. Entry of the correct PIN2 allows PIN2-protected data to be accessed over the UICC-Terminal interface. ETSI TS 102 221 [8] defines the range of "81" to "88" as key reference of the PIN2 on a multi-verification capable UICC.

#### 6.1.13.2	Conformance requirement

CR 1	Before allowing the access to PIN2 protected data, the UE shall ask the user for PIN2 verification. Only after presenting the PIN2, the user shall get access to these data.

CR 2	The VERIFY PIN function verifies the PIN2 presented by the ME to the UICC.

CR 3	Indication of a successful pin2 verification is presented to the user.

Reference:

-	ETSI TS 102 221 [8], clause 9 and clause 11.1.9;

-	TS 31.102 [19], clause 6;

-	TS 22.030 [44], clause 6.6.1.

#### 6.1.13.3	Test purpose

The purpose of this test is to verify that:

1)	the PIN2 verification procedure is performed by the ME correctly;

2)	the basic public MMI string is supported;

3)	the ME supports key references in the range of "81" to "88" as PIN2.

#### 6.1.13.4	Method of test

##### 6.1.13.4.1	Initial conditions

The values of the Default UICC as defined in clause 4.5.2 of the present document are used with the following exceptions:

The UICC shall be configured to use "07" as the reference of the PIN and "87" as reference of the PIN2 with the following values:

PIN

Key reference:	07

Logically:			8642

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 38 | 36 | 34 | 32 | FF | FF | FF | FF |

Unblock PIN

Key reference:	07

Logically:			64534231

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 36 | 34 | 35 | 33 | 34 | 32 | 33 | 31 |

PIN2

Key reference:	87

Logically:			9753

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 39 | 37 | 35 | 33 | FF | FF | FF | FF |

Unblock PIN2

Key reference:	87

Logically:

57687980

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 35 | 37 | 36 | 38 | 37 | 39 | 38 | 30 |

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case with PIN enabled.

NOTE:	To perform the UPDATE FDN data (as described in the procedure below), the default FDN UICC may be used. In this case the exceptions mentioned above shall apply.

##### 6.1.13.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Run initial activation. |  |  |  |
| 2 | USER > UE | When the UE is in the "PIN check" mode, correct PIN shall be entered. |  |  |  |
| 3 |  | The access to a PIN2 protected data field shall be performed (e.g. UPDATE FDN) |  |  |  |
| 4 |  | When the UE is in the "PIN2 check" mode, the sequence "9753#" shall be entered. |  |  |  |
|  | UE | VERIFY PIN | This is verifiable only if A.2/x is supported | CR 1 | A.2/1 OR A.2/2 |
|  | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be successful | CR 2 |  |

#### 6.1.13.5	Acceptance criteria

CR  1 is explicitly verified at step 4 via supported options A.2/1 OR A2/2, ensuring that the VERIFY PIN command is correctly sent via the ME to the UICC, with parameter P2 = "87".

CR 2 is verified on the UE. CR 2 is met if an indication is given to the user showing that this procedure was executed successfully, e.g. by displaying "OK".

### 6.1.14	Change of PIN2 on multi-verification capable UICCs

#### 6.1.14.1	Definition and applicability

The PIN2 may be changed by the user, by entering the old and new PIN2. The length of the PIN2 is between 4 and 8 digits. ETSI TS 102 221 [5] defines the range of "81" to "88" as key reference of the PIN2 on a multi-verification capable UICC.

#### 6.1.14.2	Conformance requirement

CR 1	The UE shall support the change of PIN2 procedure as defined in ETSI TS 102 221 [5], clause 11.1.10.

CR 2	Indication of a successful PIN2 change is presented to user, and only the new pin2 shall be accepted for further PIN verification.

Reference:

-	ETSI TS 102 221 [8], clauses 9 and 11.1.10;

-	TS 31.102 [19], clause 6;

-	TS 22.030 [44], clause 6.6.2.

#### 6.1.14.3	Test purpose

The purpose of this test is to verify that:

1)	the PIN2 substitution procedure is performed correctly by the UE.

2)	the basic public MMI string is supported.

3)	the UE supports key references in the range of "81" to "88" as PIN2.

#### 6.1.14.4	Method of test

##### 6.1.14.4.1	Initial conditions

The values of the Default UICC as defined in clause 4.5.2 of the present document are used  with the following exceptions:

The UICC shall be configured to use "07" as the reference of the PIN and "87" as reference of the PIN2 with the following values:

PIN

Key reference:	07

Logically:

8642

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 38 | 36 | 34 | 32 | FF | FF | FF | FF |

Unblock PIN

Key reference:	07

Logically:

64534231

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 36 | 34 | 35 | 33 | 34 | 32 | 33 | 31 |

PIN2

Key reference:	87

Logically:

9753

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 39 | 37 | 35 | 33 | FF | FF | FF | FF |

Unblock PIN2

Key reference:	87

Logically:

57687980

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 35 | 37 | 36 | 38 | 37 | 39 | 38 | 30 |

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case with PIN enabled and runs an initial activation with the correct PIN entered.

NOTE:	To perform the UPDATE FDN data (as described in the procedure below), the default FDN UICC may be used. In this case the above-mentioned exceptions shall apply.

##### 6.1.14.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | USER > UE | After USIM initialisation is completed, enter "**042*9753*12345678*12345678#" or initiate an equivalent MMI dependent procedure to change the PIN2 from '9753' to '12345678' |  |  |  |
|  | UE > UICC | CHANGE PIN | This is verifiable only if A.2/x is supported. | CR 1 | A.2/1 OR A.2/2 |
|  | USIM> UE | Check Status word |  |  |  |
|  | UE | An indication is given to the user showing whether this procedure was successful | This procedure shall be successful | CR 2 |  |
| 2 | UE | UE is powered off and on and the correct PIN is entered. |  |  |  |
| 3 |  | The access to a PIN2 protected data field shall be performed (e.g. UPDATE FDN) |  |  |  |
| 4 | USER > UE | When the UE is in the "PIN2 check" mode, incorrect sequence "9753#" shall be entered. |  |  |  |
|  | UE > UICC | VERIFY PIN | This is verifiable only if A.2/x is supported. | CR 1 | A.2/1 OR A.2/2 |
|  | USIM > UE | Check Status word |  |  |  |
|  | UE | An indication is given to the user showing whether this procedure was successful | This procedure shall be unsuccessful | CR 2 |  |
| 5 | UE | The UE is powered off and on. |  |  |  |
| 6 |  | The access to a PIN2 protected data field shall be performed (e.g. UPDATE FDN) |  |  |  |
| 7 | USER > UE | When the UE is in the "PIN check" mode, the sequence "12345678#" shall be entered. |  |  |  |
|  | UE > UICC | VERIFY PIN | This is verifiable only if A.2/x is supported. | CR 1 | A.2/1 OR A2/2 |
|  | USIM> UE | Check Status word |  |  |  |
|  | UE | An indication is given to the user showing whether this procedure was successful | This procedure shall be successful | CR 2 |  |

#### 6.1.14.5	Acceptance criteria

CR 1	is explicitly verified (i) at step 1 by analysing the CHANGE PIN command sent from ME to UICC with the parameter P2 set to "87" (ii) at step 4 and step 7 by analysing the VERIFY PIN sent via the ME to the UICC (via any supported options A.2/1 OR A2/2).

CR 2	is verified (i) at step 1 on UE by ensuring UE shall give an indication of a successful execution of the command and new PIN is accepted, (ii) at step 4 on UE by ensuring UE shall give an indication that execution of the command failed, (iii) at step 7 on UE by ensuring UE shall give an indication of a successful execution of the command.

### 6.1.15	Unblock PIN2 on multi-verification capable UICCs

#### 6.1.15.1	Definition and applicability

After three consecutive wrong entries of the PIN2, the PIN2 shall become blocked. The Unblock PIN2 command is used to unblock the PIN2. This function may be performed whether or not the PIN2 is blocked. ETSI TS 102 221 [5] defines the range of "81" to "88" as key reference of the PIN2 on a multi-verification capable UICC.

#### 6.1.15.2	Conformance requirement

CR 1	The UE shall support the Unblock PIN2 command, as defined in ETSI TS 102 221 [5], clause 11.1.13.

CR 2	Indication of a successful unblock PIN2 is presented to user

CR 3	Only the new pin2 shall be accepted for further PIN2 verification.

CR 4	Indication of a successful and unsuccessful pin2 verification is presented to user and only the new pin2 shall be accepted for further PIN2 verification.

Reference:

-	ETSI TS 102 221 [8], clauses 9 and 11.1.13;

-	TS 31.102 [19], clause 6;

-	TS 22.030 [44], clause 6.6.3.

#### 6.1.15.3	Test purpose

The purpose of this test is to verify that:

1)	the PIN2 unblocking procedure is performed correctly.

2)	the basic public MMI string is supported.

3)	the UE supports key references in the range of "81" to "88" as PIN2.

#### 6.1.15.4	Method of test

##### 6.1.15.4.1	Initial conditions

The values of the Default UICC as defined in clause 4.5.2 of the present document are used with the following exceptions:

The UICC shall be configured to use "07" as the reference of the PIN and "87" as reference of the PIN2 with the following values:

PIN

Key reference:	07

Logically:

8642

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 38 | 36 | 34 | 32 | FF | FF | FF | FF |

Unblock PIN

Key reference:	07

Logically:

64534231

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 36 | 34 | 35 | 33 | 34 | 32 | 33 | 31 |

PIN2

Key reference:	87

Logically:

9753

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 39 | 37 | 35 | 33 | FF | FF | FF | FF |

Unblock PIN2

Key reference:	87

Logically:

57687980

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 35 | 37 | 36 | 38 | 37 | 39 | 38 | 30 |

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case with PIN enabled.

NOTE:	To perform the UPDATE FDN data (as described in the procedure below), the default FDN UICC may be used. In this case the above-mentioned exceptions shall apply.

##### 6.1.15.4.2	Procedure

Sequence A:

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Run initial activation of the UE and enter the correct PIN |  |  |  |
| 2 | USER > UE | After USIM initialisation is completed, enter "**052*57687980*1234*1234#" |  |  |  |
|  | UE > UICC | UNBLOCK PIN | This is verifiable only if A.2/x is supported. | CR 1 | A.2/1 OR A.2/2 |
|  | USIM > UE | Check Status word |  |  |  |
|  | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be successful | CR 2 |  |
| 3 | UE | The UE is powered off and on and the correct PIN is entered. |  |  |  |
| 4 |  | The access to a PIN2 protected data field shall be performed (e.g. UPDATE FDN) |  |  |  |
| 5 | USER > UE | Enter the new PIN2: "1234#" |  |  |  |
|  | UE > UICC | VERIFY PIN | This is verifiable only if A.2/x is supported. | CR 3 | A.2/1 OR A.2/2 |
|  | USIM > UE | Check Status word |  |  |  |
|  | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be successful | CR 4 |  |
| 6 | UE | The UE is powered off and on and the correct PIN is entered. |  |  |  |
| 7 |  | The access to a PIN2 protected data field shall be performed (e.g. UPDATE FDN) |  |  |  |
| 8 | USER > UE | Enter a wrong PIN2 three times. |  |  |  |
|  | UE > UICC | VERIFY PIN | This is repeated 3 times, and is verifiable only if A.2/x is supported. | CR 3 | A.2/1 OR A.2/2 |
|  | USIM> UE | Check Status word |  |  |  |
|  | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be unsuccessful | CR 4 |  |
| 9 | USER > UE | Enter "**052*57687980*9753*9753#". |  |  |  |
|  | UE > UICC | UNBLOCK PIN | This is verifiable only if A.2/x is supported. | CR 1 | A.2/1 OR A.2/2 |
|  | USIM> UE | Check Status word |  |  |  |
|  | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be successful | CR 2 |  |
| 10 | UE | The UE is powered off and on and the correct PIN is entered. |  |  |  |
| 11 |  | The access to a PIN2 protected data field shall be performed (e.g. UPDATE FDN) |  |  |  |
| 12 | USER > UE | Enter the new PIN2: "9753#" |  |  |  |
|  | UE > UICC | VERIFY PIN | This is verifiable only if A.2/x is supported. | CR 3 | A.2/1 OR A.2/2 |
|  | USIM > UE | Check Status word |  |  |  |
|  | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be successful | CR 4 |  |

Sequence B:

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Run initial acvtivation of the UE and enter the correct PIN |  |  |  |
| 2 | USER > UE | Enter a wrong PIN2 three times. |  |  |  |
| 3 | USER > UE | The user shall initiate an MMI dependent procedure to unblock the PIN2 with unblock code '57687980' and a new PIN "9753" |  |  |  |
|  | UE > UICC | UNBLOCK PIN | This is verifiable only if A.2/x is supported. | CR 1 | A.2/1 OR A.2/2 |
|  | USIM> UE | Check Status word |  |  |  |
|  | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be successful | CR 2 |  |
| 4 | UE | The UE is powered off and on and the correct PIN is entered. |  |  |  |
| 5 |  | The access to a PIN2 protected data field shall be performed (e.g. UPDATE FDN) |  |  |  |
| 6 | USER > UE | Enter the new PIN2: "9753#" |  |  |  |
|  | UE > UICC | VERIFY PIN | This is verifiable only if A.2/x is supported. | CR 3 | A.2/1 OR A.2/2 |
|  | USIM > UE | Check Status word |  |  |  |
|  | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be successful | CR 4 |  |

#### 6.1.15.5	Acceptance criteria

Sequence A:

CR 1 is explicitly verified at step 2 and step 9 by analysing the UNBLOCK PIN command sent via ME to the UICC, with parameter P2 = "87" (via any supported options A.2/1 OR A2/2).

CR 2 is verified at step 2 and step 9 on the UE by ensuring UE shall give an indication of a successful execution of the command and new PIN2 is accepted.

CR 3 is explicitly verified at step 5, step 8 and step 12 by analysing the Status word for both successful and unsuccessful PIN2 entry.

CR 4 is verified at (i) step 5 and step 12 on the UE by ensuring UE shall give an indication of a successful execution of the command and PIN2 is accepted, (ii) at step 8 on UE by ensuring UE shall give an indication that execution of the command failed.

Sequence B:

CR 1 is explicitly verified at step 3 by analysing the UNBLOCK PIN command sent via ME to the UICC, with parameter P2 = "87" (via any supported options A.2/1 OR A2/2) after UE indicates that PIN has been blocked in step2.

CR 2 is verified at step 3 on the UE by ensuring UE shall give an indication following a successful execution of the command and new PIN2 is accepted.

CR 3 is explicitly verified at step 6 by analysing the Status word for successful PIN2 entry.

CR 4 is verified at step 6 on the UE by ensuring UE shall give an indication following a successful execution of the command that PIN2 is accepted.

### 6.1.16	Replacement of PIN with key reference "07"

#### 6.1.16.1	Definition and applicability

The Universal PIN may be used to replace a PIN used to authenticate the user to the UICC for security. In this case entry of the correct Universal PIN allows PIN-protected data to be accessed over the UICC-Terminal interface.

#### 6.1.16.2	Conformance requirement

CR 1	The UE shall support the usage of the Universal PIN as replacement PIN and the replacement procedure as defined in ETSI TS 102 221 [5], clause 11.1.11.

CR 2	The UE shall support the procedure to disable the replacement defined in ETSI TS 102 221 [5], clause 11.1.12.

CR 3	Only the new pin shall be accepted for further PIN verification.

CR 4	Indication of a successful and unsuccessful pin verification is presented to the user and only the new pin shall be accepted for further PIN verification.

Reference:

-	ETSI TS 102 221 [8], clauses 9, 11.1.11 and 11.1.12;

-	TS 31.102 [19], clause 6.

#### 6.1.16.3	Test purpose

The purpose of this test is to verify that:

1)	the PIN replacement is supported by the UE correctly.

2)	the PIN replacement procedure is performed by the UE correctly.

3)	the procedure to disable the PIN replacement is performed by the UE correctly.

4)	the UE supports key references in the range of "01" to "08" as PIN.

#### 6.1.16.4	Method of test

##### 6.1.16.4.1	Initial conditions

The values of the Default UICC as defined in clause 4.5.2 of the present document are used with the following exceptions:

The UICC shall be configured to use "07" as the reference of the PIN and "87" as reference of the PIN2 with the following values:

PIN

Key reference:	07

Logically:

8642

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 38 | 36 | 34 | 32 | FF | FF | FF | FF |

Unblock PIN

Key reference:	07

Logically:

64534231

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 36 | 34 | 35 | 33 | 34 | 32 | 33 | 31 |

PIN2

Key reference:	87

Logically:

9753

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 39 | 37 | 35 | 33 | FF | FF | FF | FF |

Unblock PIN2

Key reference:	87

Logically:

57687980

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 35 | 37 | 36 | 38 | 37 | 39 | 38 | 30 |

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case with PIN enabled.

##### 6.1.16.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Run initial activation. |  |  |  |
| 2 | USER > UE | When the UE is in the "PIN check" mode, the sequence "8642#" shall be entered. |  |  |  |
|  | UE > UICC | VERIFY PIN | This is verifiable only if A.2/x is supported. |  | A.2/1 OR A.2/2 |
|  | USIM > UE | Check Status word |  |  |  |
|  | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be successful |  |  |
| 3 | USER > UE | The user shall initiate an MMI dependent procedure to replace the PIN by the Universal PIN. |  |  |  |
|  | UE > UICC | DISABLE PIN | This is verifiable only if A.2/x is supported. | CR 1 |  |
|  | USIM > UE | Check Status word |  |  |  |
| 4 | UE > USER | The UE is powered off and on. |  |  |  |
| 5 | USER > UE | When the UE is in the "PIN check" mode, the sequence "8642#" shall be entered. |  |  |  |
|  | UE > UICC | VERIFY PIN | This is verifiable only if A.2/x is supported. | CR 3 | A.2/1 OR A.2/2 |
|  | USIM > UE | Check Status word |  |  |  |
|  | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be unsuccessful | CR 4 |  |
| 6 |  | The correct Universal PIN is entered. |  |  |  |
|  | UE > UICC | VERIFY PIN | This is verifiable only if A.2/x is supported. | CR 3 | A.2/1 OR A.2/2 |
|  | USIM > UE | Check Status word |  |  |  |
|  | UE | An indication is given to the user showing whether this procedure was successful | This procedure shall be successful | CR 4 |  |
| 7 |  | Initiate an MMI dependent procedure to disable the replacement of the PIN by the Universal PIN |  |  |  |
|  | UE > UICC | ENABLE PIN | This is verifiable only if A.2/x is supported. | CR 2 | A.2/1 OR A.2/2 |
|  | USIM > UE | Check Status word |  |  |  |
| 8 | UE > USER | The UE is powered off and on. |  |  |  |
| 9 | USER > UE | When the UE is in the "PIN check" mode, the sequence "2839#" shall be entered. |  |  |  |
|  | UE > UICC | VERIFY PIN | This is verifiable only if A.2/x is supported. | CR 3 | A.2/1 OR A.2/2 |
|  | USIM > UE | Check Status word |  |  |  |
|  | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be unsuccessful | CR 4 |  |
| 10 | USER > UE | When the UE is in the "PIN check" mode correct PIN is entered. |  |  |  |
|  | UE > UICC | VERIFY PIN | This is verifiable only if A.2/x is supported. | CR 3 | A.2/1 OR A.2/2 |
|  | USIM > UE | Check Status word |  |  |  |
|  | UE > USER | An indication is given to the user showing whether this procedure was successful | This procedure shall be successful | CR 4 |  |

#### 6.1.16.5	Acceptance criteria

CR 1 is explicitly verified at step 3 by analysing the DISABLE PIN command sent via the ME to the UICC, with parameter P1 = "91" and P2 = "07" (via any supported options A.2/1 OR A2/2).

CR 2 is explicitly verified at step 7 by analysing the ENABLE PIN command sent via the ME to the UICC, with parameter P2 = "07" (via any supported options A.2/1 OR A2/2)

CR 3 is explicitly verified at (i) step 5 by analysing the status word indicating that the PIN has not been accepted for the VERIFY PIN command sent by the ME to the UICC, with parameter P2 = "11", (ii) step 6 by analysing the status word indicating that the PIN has been accepted for the VERIFY PIN command sent by the ME to the UICC, with parameter P2 = "11", (iii) step 9 by analysing the status word indicating that the PIN has not been accepted for the VERIFY PIN command sent by the ME to the UICC, with parameter P2 = "07", (ii) step 10 by analysing the status word indicating that the PIN has been accepted for the VERIFY PIN command sent by the ME to the UICC, with parameter P2 = "07".

CR 4 is verified (i) at step 5 and 9 by ensuring that the UE gives an indication of an unsuccessful execution of the command and PIN is not accepted, (ii) at step 6 and 10 by ensuring that the UE gives an indication of a successful execution of the command and PIN is accepted.

## 6.2	Fixed Dialling Numbers (FDN) handling

### 6.2.1	ME and USIM with FDN enabled, EFADN readable and updateable

GERAN/UTRAN test - not applicable

### 6.2.2	ME and USIM with FDN disabled

GERAN/UTRAN test - not applicable

### 6.2.3	Enabling, disabling and updating of FDN

GERAN/UTRAN test - not applicable

### 6.2.4	ME and USIM with FDN enabled, EFADN readable and updateable (Rel-4 and onwards)

GERAN/UTRAN test - not applicable

## 6.3	Void

## 6.4	Advice of charge (AoC) handling

### 6.4.1	AoC not supported by USIM

GERAN/UTRAN test - not applicable

### 6.4.2	Maximum frequency of ACM updating

GERAN/UTRAN test - not applicable

### 6.4.3	Call terminated when ACM greater than ACMmax

GERAN/UTRAN test - not applicable

### 6.4.4	Response codes of increase command of ACM

GERAN/UTRAN test - not applicable

# 7	PLMN related tests

## 7.1	FPLMN handling

### 7.1.1	Adding FPLMN to the Forbidden PLMN list

GERAN/UTRAN test - not applicable

### 7.1.2	UE updating forbidden PLMNs

GERAN/UTRAN test - not applicable

### 7.1.3	UE deleting forbidden PLMNs

GERAN/UTRAN test - not applicable

### 7.1.4	Adding FPLMN to the forbidden PLMN list when accessing E-UTRAN

#### 7.1.4.1	Definition and applicability

A list of forbidden PLMNs stored in the USIM and providing storage for at least 4 entries is managed by the UE. In automatic PLMN selection mode the UE controls registration attempts to appropriate networks with respect to this list of forbidden PLMNs. As a result of a registration reject with the cause "PLMN not allowed" the UE stores the PLMN which rejected the update request in the USIM.

#### 7.1.4.2	Conformance requirement

CR 1	In automatic PLMN selection mode the UE shall only attempt an ATTACH REQUEST during registration on E-UTRAN/EPS if it receives a BCCH containing a PLMN (MCC, MNC) that is not indicated in the EFFPLMN in the USIM

Reference:

-	TS 22.011 [18], clause 2.3;

-	TS 31.102 [19], clauses 5.1.1 and 5.2.7.

CR 2	After receipt of an ATTACH REJECT message during registration on E-UTRAN/EPS with the EMM cause "PLMN not allowed" the ME shall update the EFFPLMN in the USIM.

Reference:

-	TS 22.011 [18], clause 3.2.2 2;

-	TS 31.102 [19], clauses 5.1.1 and 5.2.7

CR 3	After receipt of an ATTACH REJECT message during registration on E-UTRAN/EPS with the EMM cause "PLMN not allowed" the ME shall update the EFEPSLOCI in the USIM.

Reference:

-	TS 24.301 [21], clause 5.5.1.2.5;

-	TS 31.102 [19], clauses 5.1.2 and 4.2.9.1.

CR 4	After receipt of an ATTACH REJECT message during registration on E-UTRAN/EPS with the After registration on E-UTRAN/EPS the USIM shall contain the correct GUTI and TAI received by the UE.

Reference:

-	TS 31.102 [19], clauses 5.1.2 and 4.2.9.1;

-	TS 21.111 [20], clause 10.1.

#### 7.1.4.3	Test purpose

The purpose of this test is to verify that:

1)	in automatic PLMN selection mode the UE does not attempt to access PLMNs stored in EFFPLMN on the USIM;

2)	the EFFPLMN is correctly updated by the ME after receipt of an ATTACH REJECT message with cause "PLMN not allowed" during registration;

3)	the EFEPSLOCI has been correctly updated by the ME during registration.

#### 7.1.4.4	Method of test

##### 7.1.4.4.1	Initial conditions

The values of the FDN UICC as defined in clause 4.5.3 of the present document are used with EFIMSI as defined in clause 4.6.2.

The defined UICC/USIM configuration defined for this test case shall be used and made available on the UE.

Ensure that the automatic PLMN selection mode is set and that the UE has installed and is using the UICC/USIM configuration defined for this test case and runs an initial activation when executing the test procedure.

##### 7.1.4.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | TT | Activate the RF output on the BCCH with: - TAI (MCC/MNC/TAC): 234/002/0001 - Access control: unrestricted | The UE does not send an ATTACH REQUEST message as the PLMN provided by the TT is listed in EFFPLMN | CR 1 |  |
| 2 | TT | Stop RF output on the BCCH | The UE starts a reselection procedure |  |  |
| 3 | TT | Resume the RF output on the BCCH with: - TAI (MCC/MNC/TAC): 234/003/0001 - Access control: unrestricted | The UE does not send an ATTACH REQUEST message as the PLMN provided by the TT is it is listed in EFFPLMN | CR 1 |  |
| 4 | TT | Stop RF output on the BCCH | The UE starts a reselection procedure |  |  |
| 5 | TT | Resume the RF output on the BCCH with: - TAI (MCC/MNC/TAC): 234/004/0001 - Access control: unrestricted | The UE does not send an ATTACH REQUEST message as the PLMN provided by the TT is it is listed in EFFPLMN | CR 1 |  |
| 6 | TT | Stop RF output on the BCCH | The UE starts a reselection procedure |  |  |
| 7 | TT | Resume the RF output on the BCCH with: - TAI (MCC/MNC/TAC): 234/005/0001 - Access control: unrestricted | The UE does not send an ATTACH REQUEST message as the PLMN provided by the TT is it is listed in EFFPLMN | CR 1 |  |
| 8 | TT | Stop RF output on the BCCH | The UE starts a reselection procedure |  |  |
| 9 | TT | Resume the RF output on the BCCH with: - TAI (MCC/MNC/TAC): 234/007/0001 - Access control: unrestricted | The UE tries to connect to the PLMN provided by the TT | CR 1 |  |
| 10 | UE > TT | Send RRC CONNECTION REQUEST / RRC CONNECTION REQUEST-NB | The TT responds with a RRC CONNECTION SETUP / RRC CONNECTION SETUP-NB |  |  |
| 11 | UE > TT | Send RRC CONNECTION SETUP COMPLETE / RRC CONNECTION SETUP COMPLETE-NB |  |  |  |
| 12 | UE > TT | Send ATTACH REQUEST | The TT performs an authentication and starts NAS integrity protection |  |  |
| 13 | TT > UE | Send ATTACH REJECT message with cause 'PLMN not allowed' | The UE updates EFFPLMN The UE updates EFEPSLOCI | CR 2 CR 3 | A.2/1 OR A.2/2 |
| 14 | TT > UE | Send RRC CONNECTION RELEASE / RRC CONNECTION RELEASE-NB |  |  |  |
| 15 | TT | Stop RF output on the BCCH |  |  |  |
| 16 | UE | Disable signalling on RAN |  |  |  |
| 17 | TT | Read out EFEPSLOCI | EFEPSLOCI content can be verified by the TT | CR 3 | A.2/1 OR A.2/2 |
| 18 | UE | Enable signalling on RAN | The UE starts a reselection procedure |  |  |
| 19 | TT | Resume the RF output on the BCCH with: - TAI (MCC/MNC/TAC): 234/008/0001 - Access control: unrestricted | The UE tries to connect to the PLMN provided by the TT | CR 1 |  |
| 20 | UE > TT | Send RRC CONNECTION REQUEST / RRC CONNECTION REQUEST-NB | The TT responds with a RRC CONNECTION SETUP / RRC CONNECTION SETUP-NB |  |  |
| 21 | UE > TT | Send RRC CONNECTION SETUP COMPLETE / RRC CONNECTION SETUP COMPLETE-NB |  |  |  |
| 22 | UE > TT | Send ATTACH REQUEST | The TT performs an authentication and starts NAS integrity protection |  |  |
| 23 | TT > UE | Send ATTACH ACCEPT message with: - TAI (MCC/MNC/TAC): 234/008/0001 - GUTI (as generated from TT) | The UE responds with an ATTACH COMPLETE message | CR 4 |  |
|  | UE | Update EFEPSLOCI |  | CR 3 | A.2/1 OR A.2/2 |
| 24 | TT > UE | Send a RRC CONNECTION RELEASE / RRC CONNECTION RELEASE-NB |  |  |  |

#### 7.1.4.4	Acceptance criteria

CR 1 is met if the UE is not sending an ATTACH REQUEST in steps 1), 3), 5) and 7) but in step 9) and 19) as indicated by correctly interpreted EFFPLMN content. During test execution the TT shall provide status information related to the ATTACH REQUEST.

CR 2 shall be verified in step 13). Test environments not capable of evaluating the content of EFFPLMN while the common test procedure is executed, may verify the correct update of EFFPLMN by reading out the file at the end of the test execution. All entries generated during test execution shall be available and comply to the EF listings below, else the conformance requirement is not met.

CR 3 shall be verified in step 17). Test environments not capable of evaluating the content of EFEPSLOCI while the common test procedure is executed, may verify the correct update of EFEPSLOCI by reading out the file at the end of the test execution. All entries generated during test execution shall be available and comply to the EF listings below, else the conformance requirement is not met.

CR 4 is met if the GUTI derived from EFEPSLOCI matches the GUTI generated from the TT in step 13).

EFEPSLOCI (EPS Location Information)

Logically:

GUTI:								as generated from TT in step 13)

Last visited registered TAI:		234/007/0001

EPS update status:				updated

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 0B | F6 | xx | xx | xx | xx | xx | xx | xx | xx | xx | xx |
|  | B13 | B14 | B15 | B16 | B17 | B18 |
|  | 32 | 74 | 00 | 00 | 01 | 00 |

The EFs of the USIM read after finishing the common test procedure shall contain the following values:

EFFPLMN (Forbidden PLMNs)

Logically:

PLMN1:	234 002 (MCC MNC)

PLMN2:	234 003

PLMN3:	234 004

PLMN4:	234 005

PLMN5:	234 006

PLMN6:	234 007

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 32 | 24 | 00 | 32 | 34 | 00 | 32 | 44 | 00 | 32 | 54 | 00 |
|  | B13 | B14 | B15 | B16 | B17 | B18 |
|  | 32 | 64 | 00 | 32 | 74 | 00 |

EFEPSLOCI (EPS Location Information)

Logically:

GUTI:								as generated from TT in step 13)

Last visited registered TAI:		234/008/0001

EPS update status:				updated

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 0B | F6 | xx | xx | xx | xx | xx | xx | xx | xx | xx | xx |
|  | B13 | B14 | B15 | B16 | B17 | B18 |
|  | 32 | 84 | 00 | 00 | 01 | 00 |

### 7.1.5	UE updating forbidden PLMNs when accessing E-UTRAN

#### 7.1.5.1	Definition and applicability

A list of forbidden PLMNs stored in the USIM provides storage for at least 4 entries, and is managed by the UE. In automatic PLMN selection mode the UE controls registration attempts to appropriate networks with respect to this list of forbidden PLMNs. As a result of a registration reject with the cause "PLMN not allowed" the UE stores the PLMN which rejected the update request in the USIM.

#### 7.1.5.2	Conformance requirement

CR 1	After receipt of an ATTACH REJECT message during registration with the cause "PLMN not allowed" the ME shall update the EFFPLMN in the USIM.

Reference:

-	TS 22.011 [18], clause 3.2.2 4;

-	TS 31.102 [19], clauses 5.1.1 and 5.2.7

#### 7.1.5.3	Test purpose

The purpose of this test is to verify that the UE correctly updates the EFFPLMN, i.e. fill up existing gaps in the elementary file before overwriting any existing entries.

#### 7.1.5.4	Method of test

##### 7.1.5.4.1	Initial conditions

The values of the E-UTRAN/EPC UICC as defined in clause 4.5.4 of the present document are used with EFIMSI as defined in clause 4.6.2.

The defined UICC/USIM configuration defined for this test case shall be used and made available on the UE.

Ensure that the automatic PLMN selection mode is set and that the UE has installed and is using the UICC/USIM configuration defined for this test case in TS 31.121 [2] and runs an initial activation when executing the test procedure.

##### 7.1.5.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE > TT | Send RRC CONNECTION REQUEST / RRC CONNECTION REQUEST-NB | The TT responds with a RRC CONNECTION SETUP / RRC CONNECTION SETUP-NB |  |  |
| 2 | UE > TT | Send RRC CONNECTION SETUP COMPLETE / RRC CONNECTION SETUP COMPLETE-NB |  |  |  |
| 3 | UE > TT | Send ATTACH REQUEST | The TT performs an authentication and starts NAS integrity protection |  |  |
| 4 | TT > UE | Send ATTACH REJECT message with cause 'PLMN not allowed' | The UE updates EFFPLMN | CR 1 | A.2/1 OR A.2/2 |
| 5 | TT > UE | Send RRC CONNECTION RELEASE / RRC CONNECTION RELEASE-NB |  |  |  |
| 6 | USER > UE | UE is soft powered down |  |  |  |

#### 7.1.5.5	Acceptance criteria

CR 1 shall be verified in step 4 after receiving ATTACH REJECT. Test environments not capable of evaluating the content of EFFPLMN while the common test procedure is executed, may verify the correct update of EFFPLMN by reading out the file at the end of the test execution. All entries generated during test execution shall be available and comply to the EF listings below, else the conformance requirement is not met.

EFFPLMN (Forbidden PLMNs)

Logically:

PLMN1:	234 001 (MCC MNC)

PLMN2:	234 002

PLMN3:	234 003

PLMN4:	234 004

PLMN5:	234 005

PLMN6:	234 006

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 32 | 14 | 00 | 32 | 24 | 00 | 32 | 34 | 00 | 32 | 44 | 00 |
|  | B13 | B14 | B15 | B16 | B17 | B18 |
|  | 32 | 54 | 00 | 32 | 64 | 00 |

Or

EFFPLMN (Forbidden PLMNs)

Logically:

PLMN1:	234 001 (MCC MNC)

PLMN2:	234 003

PLMN3:	234 004

PLMN4:	234 005

PLMN5:	234 006

PLMN6:	234 002

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 32 | 14 | 00 | 32 | 34 | 00 | 32 | 44 | 00 | 32 | 54 | 00 |
|  | B13 | B14 | B15 | B16 | B17 | B18 |
|  | 32 | 64 | 00 | 32 | 24 | 00 |

### 7.1.6	UE deleting forbidden PLMNs when accessing E-UTRAN

#### 7.1.6.1	Definition and applicability

In manual PLMN selection mode the UE allows registration attempts to all available PLMNs, including forbidden PLMNs (as indicated by the forbidden PLMN list on the USIM). As a result of a successful registration procedure onto a PLMN which is in the forbidden PLMN list, the forbidden PLMN list is automatically updated by the UE.

#### 7.1.6.2	Conformance requirement

CR 1	In manual PLMN selection mode the UE shall be able to perform a ATTACH attempt during registration to a PLMN which is in the forbidden PLMN list.

Reference:

-	TS 22.011 [18], clause 3.2.2.2;

-	TS 31.102 [19], clauses 5.1.1 and 5.2.7.

CR 2	After receipt of ATTACH ACCEPT message during registration the UE shall delete the forbidden PLMN from the forbidden PLMN list.

Reference:

-	TS 22.011 [18], clause 3.2.2.4;

#### 7.1.6.3	Test purpose

The purpose of this test is to verify that:

1)	the UE is able to perform an ATTACH REQUEST during registration on a forbidden PLMN in manual PLMN selection mode.

2)	the UE after a successful registration attempt deletes the PLMN in the EFFPLMN on the USIM.

#### 7.1.6.4	Method of test

##### 7.1.6.4.1	Initial conditions

The values of the E-UTRAN/EPC UICC as defined in clause 4.5.4 of the present document are used with EFIMSI as defined in clause 4.6.2.

The defined UICC/USIM configuration defined for this test case shall be used and made available on the UE.

The TT (E-USS or NB-SS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):	234/005/0001.

-	Access control:	unrestricted.

Ensure that the manual PLMN selection mode is set and that the UE has installed and is using the UICC/USIM configuration defined for this test case in 31.121 [2] and runs an initial activation when executing the test procedure.

##### 7.1.6.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | USER > UE | PLMN with MCC/MNC 234/005 is manually selected. |  |  |  |
| 2 | UE > TT | RRC CONNECTION REQUEST / RRC CONNECTION REQUEST-NB | The TT responds with a RRC CONNECTION SETUP / RRC CONNECTION SETUP-NB |  |  |
| 3 | UE > TT | RRC CONNECTION SETUP COMPLETE / RRC CONNECTION SETUP COMPLETE-NB |  |  |  |
| 4 | UE > TT | ATTACH REQUEST | The TT performs an authentication and starts NAS integrity protection | CR 1 |  |
| 5 | TT > UE | ATTACH ACCEPT message with: - TAI (MCC/MNC/TAC): 234/005/ 0001 - GUTI (as generated from TT) | The UE responds with an ATTACH COMPLETE message |  |  |
| 6 | UE > USIM | - UDATE EFFPLMN, EFEPSLOCI |  | CR 2 | A.2/1 OR A.2/2 |
| 7 | TT > UE | RRC CONNECTION RELEASE / RRC CONNECTION RELEASE-NB |  |  |  |
| 8 | UE | The UE is soft powered down. |  |  |  |

#### 7.1.6.5	Acceptance criteria

CR 1 is met if the UE is sending ATTACH REQUEST in step 4) of Sequence A. TT shall provide status information.

CR 2 shall be explicitly verified in step 6). Test environments not capable of evaluating the content of EFFPLMN, EFEPSLOCI while the common test procedure is executed, may verify the correct update of EFFPLMN , EFEPSLOCI by reading out the file at the end of the test execution. All entries generated during test execution shall be available and comply to the EF listings below, else the conformance requirement is not met.

EFFPLMN (Forbidden PLMNs)

Logically:

PLMN1:	empty

PLMN2:	empty

PLMN3:	empty

PLMN4:	empty

PLMN5:	empty

PLMN6:	empty

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | FF | FF | FF | FF | FF | FF | FF | FF | FF | FF | FF | FF |
|  | B13 | B14 | B15 | B16 | B17 | B18 |
|  | FF | FF | FF | FF | FF | FF |

EFEPSLOCI (EPS Location Information)

Logically:

GUTI:								as generated from TT in step 5)

Last visited registered TAI:		234/005/0001

EPS update status:				updated

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 0B | F6 | xx | xx | xx | xx | xx | xx | xx | xx | xx | xx |
|  | B13 | B14 | B15 | B16 | B17 | B18 |
|  | 32 | 54 | 00 | 00 | 01 | 00 |

### 7.1.7	Updating the Forbidden PLMN list after receiving non-integrity protected reject message – UTRAN

UTRAN test - not applicable

### 7.1.8	Updating the Forbidden PLMN list after receiving non-integrity protected reject message – E-UTRAN

#### 7.1.8.1	Definition and applicability

A list of forbidden PLMNs stored in the USIM and providing storage for at least 4 entries is managed by the UE. In automatic PLMN selection mode the UE controls registration attempts to appropriate networks with respect to this list of forbidden PLMNs. As a result of a registration reject with the cause "PLMN not allowed" the UE stores the PLMN which rejected the update request in the USIM.

#### 7.1.8.2	Conformance requirement

CR 1	After receipt of a not integrity-protected ATTACH REJECT message during registration with the cause "PLMN not allowed" the ME shall update the EFFPLMN in the USIM thereafter that VPLMN will not be accessed by the MS in automatic mode:

-		if the MS is not configured to use timer T3245, and the MS maintains a list of PLMN-specific attempt counters and the value of the PLMN-specific attempt counter for that VPLMN is equal to the MS implementation specific maximum value or;

-	if the MS is not configured to use timer T3245, and the MS is not maintaining a list of PLMN-specific attempt counters

Reference:

-	TS 23.122 [41], clause 3.1.

-	TS 22.011 [18], clause 3.2.2 2.3;

-	TS 31.102 [19], clauses 5.1.1 and 5.2.7.

#### 7.1.8.3	Test purpose

The purpose of this test is to verify that:

1)	in automatic PLMN selection mode and after receipt of ATTACH REJECT message with cause "PLMN not allowed" during registration that the UE correctly updates EFFPLMN, in the following cases:

-	if the UE maintains a list of PLMN-specific attempt counters, and the value of the PLMN-specific attempt counter for that VPLMN is equal to the MS implementation specific maximum value;

-	if the UE does not maintain a list of PLMN-specific attempt counters.

2)	the UE correctly updates the EFFPLMN, i.e. fill up existing gaps in the elementary file before overwriting any existing entries.

#### 7.1.8.4	Method of test

##### 7.1.8.4.1	Initial conditions

The values of the E-UTRAN/EPC UICC as defined in clause 4.5.4 of the present document are used with EFIMSI as defined in clause 4.6.2.

If option A.1/38 is supported by the UE set the implementation specific counter to small value to reduce the test execution time. Use the value set for th number of repetitions as indicated in step 7).

The defined UICC/USIM configuration defined for this test case shall be used and made available on the UE.

The TT (E-USS or NB-SS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):	234/002/0001.

-	Access control:	unrestricted.

Ensure that the automatic PLMN selection mode is set and that the UE has installed and is using the UICC/USIM configuration defined for this test case in 31.121 [2] and runs an initial activation when executing the test procedure.

##### 7.1.8.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE > TT | - Send RRC CONNECTION REQUEST / RRC CONNECTION REQUEST-NB | TT responds with RRC CONNECTION SETUP / RRC CONNECTION SETUP-NB |  |  |
| 2 | UE > TT | Send RRC CONNECTION SETUP COMPLETE / RRC CONNECTION SETUP COMPLETE-NB |  |  |  |
| 3 | UE > TT | - ATTACH REQUEST |  |  |  |
| 4 | TT > UE | - ATTACH REJECT (Cause: PLMN Not Allowed) | Not integrity protected |  |  |
| 5 | TT > UE | - Send RRC CONNECTION RELEASE / RRC CONNECTION RELEASE-NB |  |  |  |
| 6 | If the UE supports A.1/38, perform step 7) after the expiry of timer T3247, otherwise perform step 8). |  |  |  |  |
| 7 | Using the settings declared for the implementation specific counter, repeat step 1) – 6) until the PLMN-specific attempt counters has reached the maximum value for that VPLMN |  |  |  |  |
| 8 | UE > USIM | - Update EFFPLMN |  | CR 1 | A.2/1 OR A.2/2 |
| 9 | UE | - The UE is soft powered down. |  |  |  |

#### 7.1.8.5	Acceptance criteria

CR 1 shall be verified in step 8). Test environments not capable of evaluating the content of EFFPLMN while the common test procedure is executed, may verify the correct update of EFFPLMN by reading out the file at the end of the test execution. All entries generated during test execution shall be available and comply to the EF listings below, else the conformance requirement is not met.

EFFPLMN (Forbidden PLMNs)

Logically:

PLMN1:	234 001 (MCC MNC)

PLMN2:	234 002

PLMN3:	234 003

PLMN4:	234 004

PLMN5:	234 005

PLMN6:	234 006

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 32 | 14 | 00 | 32 | 24 | 00 | 32 | 34 | 00 | 32 | 44 | 00 |
|  | B13 | B14 | B15 | B16 | B17 | B18 |
|  | 32 | 54 | 00 | 32 | 64 | 00 |

Or

EFFPLMN (Forbidden PLMNs)

Logically:

PLMN1:	234 001 (MCC MNC)

PLMN2:	234 003

PLMN3:	234 004

PLMN4:	234 005

PLMN5:	234 006

PLMN6:	234 002

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 32 | 14 | 00 | 32 | 34 | 00 | 32 | 44 | 00 | 32 | 54 | 00 |
|  | B13 | B14 | B15 | B16 | B17 | B18 |
|  | 32 | 64 | 00 | 32 | 24 | 00 |

## 7.2	User controlled PLMN selector handling

### 7.2.1	UE updating the User controlled PLMN selector list

#### 7.2.1.1	Definition and applicability

The User controlled PLMN selector list gives in priority order the preferred UPLMNs on which the UE shall register. The Radio Access Technology identifier defines the Radio network in which the UE shall register. The list is stored on the USIM in the EFPLMNwACT. Update and deletion of PLMNs may be performed by the subscriber.

#### 7.2.1.2	Conformance requirement

CR 1	The UE shall correctly replace the selected UPLMN in the User controlled PLMN selector list.

Reference:

-	TS 31.102 [19], clause 5.3.6.

#### 7.2.1.3	Test purpose

The purpose of this test is to verify that the UE correctly updates the EFPLMNwACT.

#### 7.2.1.4	Method of test

##### 7.2.1.4.1	Initial conditions

The values of the Default UICC as defined in clause 4.5.2 of the present document are used.

The defined UICC/USIM configuration defined for this test case shall be used and made available on the UE.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and runs an initial activation.

##### 7.2.1.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | USER > UE | - Initiate an MMI dependent procedure to change the UPLMN | Change the second UPLMN in the User controlled PLMN selector list to MCC/MNC of 567/02, the ACT identifier shall set to UTRAN only. |  |  |
| 2 | UE > USIM | Update EFPLMNwACT |  | CR 1 | A.2/1 OR A.2/2 |
| 3 | UE | - The UE is soft powered down |  |  |  |

| Optional step if CR 1 verification cannot be performed at step 2) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 4 | UE | READ EFPLMNwACT PLMNwACT |  | CR 1 | A.2/3 |

#### 7.2.1.5	Acceptance criteria

After step b) the USIM shall contain the following values:

EFPLMNwACT (UPLMN Selector)

Logically:

1st PLMN:	244 081 (MCC MNC)

1st ACT:	UTRAN

2nd PLMN:	567 02

2nd ACT	UTRAN

3rd PLMN:	244 082

3rd ACT	UTRAN

4th PLMN:	244 082

4th ACT	GSM

5th PLMN:	244 003

5th ACT	UTRAN

6th PLMN:	244 004

6th ACT	UTRAN

7th PLMN:	244 005

7th ACT	UTRAN

8th PLMN:	244 006

8th ACT	UTRAN

9th PLMN:	244 007

9th ACT	UTRAN

10th PLMN:	244 008

10th ACT	UTRAN

11th PLMN:	244 009

11th ACT	UTRAN

12th PLMN:	244 010

12th ACT	UTRAN

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 42 | 14 | 80 | 80 | 00 | 65 | F7 | 20 | 80 | 00 | 42 | 24 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 80 | 80 | 00 | 32 | 24 | 00 | 00 | 80 | 42 | 34 | 00 | 80 |
|  | B25 | B26 | B27 | B28 | B29 | B30 | B31 | B32 | B33 | B34 | B35 | B36 |
|  | 00 | 42 | 44 | 00 | 80 | 00 | 42 | 54 | 00 | 80 | 00 | 42 |
|  | B37 | B38 | B39 | B40 | B41 | B42 | B43 | B44 | B45 | B46 | B47 | B48 |
|  | 64 | 00 | 80 | 00 | 42 | 74 | 00 | 80 | 00 | 42 | 84 | 00 |
|  | B49 | B50 | B51 | B52 | B53 | B54 | B55 | B56 | B57 | B58 | B59 | B60 |
|  | 80 | 00 | 42 | 94 | 00 | 80 | 00 | 42 | 04 | 10 | 80 | 00 |

### 7.2.2	UE recognizing the priority order of the User controlled PLMN selector list with the same access Technology

GERAN/UTRAN test - not applicable

### 7.2.3	UE recognizing the priority order of the User controlled PLMN selector list using an ACT Preference

GERAN/UTRAN test - not applicable

### 7.2.4	Void

### 7.2.5	UE updating the User controlled PLMN selector list for E-UTRAN

#### 7.2.5.1	Definition and applicability

The User controlled PLMN selector list gives in priority order the preferred UPLMNs on which the UE shall register. The Radio Access Technology identifier defines the Radio network in which the UE shall register. The list is stored on the USIM in the EFPLMNwACT. Update and deletion of PLMNs may be performed by the subscriber.

#### 7.2.5.2	Conformance requirement

CR 1	The UE shall correctly replace the selected UPLMN in the User controlled PLMN selector list.

Reference:

-	TS 31.102 [19], clause 5.3.6 and 4.2.5.

#### 7.2.5.3	Test purpose

The purpose of this test is to verify that the UE correctly updates the EFPLMNwACT.

#### 7.2.5.4	Method of test

##### 7.2.5.4.1	Initial conditions

The values of the E-UTRAN/EPC UICC as defined in clause 4.5.4 of the present document are used.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and runs an initial activation.

##### 7.2.5.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | USER > UE | - Initiate an MMI dependent procedure to change the UPLMN | Change the second UPLMN in the User controlled PLMN selector list to MCC/MNC of 567/04, the ACT identifier shall set to E-UTRAN only. |  |  |
| 2 | UE > USIM | Update EFPLMNwACT |  | CR 1 | A.2/1 OR A.2/2 |
| 3 | UE | - The UE is soft powered down |  |  |  |

| Optional step if CR 1 verification cannot be performed at step 2) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 4 | UE | READ EFPLMNwACT PLMNwACT |  | CR 1 | A.2/3 |

#### 7.2.5.5	Acceptance criteria

After step b) the USIM shall contain the following values:

EFPLMNwACT (UPLMN Selector)

Logically:

1st PLMN:	244 081 (MCC MNC)

1st ACT:	E-UTRAN

2nd PLMN:	567 04

2nd ACT	E-UTRAN

3rd PLMN:	244 083

3rd ACT:	E-UTRAN

4th PLMN:	244 082

4th ACT:	GSM

5th PLMN:	244 003

5th ACT:	E-UTRAN

6th PLMN:	244 004

6th ACT:	UTRAN

7th PLMN:	244 005

7th ACT:	UTRAN

8th PLMN:	244 081

8th ACT:	UTRAN

9th PLMN:	244 007

9th ACT:	UTRAN

10th PLMN:	244 008

10th ACT:	E-UTRAN

11th PLMN:	244 009

11th ACT:	UTRAN

12th PLMN:	244 010

12th ACT:	E-UTRAN

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 42 | 14 | 80 | 40 | 00 | 65 | F7 | 40 | 40 | 00 | 42 | 34 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 80 | 40 | 00 | 42 | 24 | 80 | 00 | 80 | 42 | 34 | 00 | 40 |
|  | B25 | B26 | B27 | B28 | B29 | B30 | B31 | B32 | B33 | B34 | B35 | B36 |
|  | 00 | 42 | 44 | 00 | 80 | 00 | 42 | 54 | 00 | 80 | 00 | 42 |
|  | B37 | B38 | B39 | B40 | B41 | B42 | B43 | B44 | B45 | B46 | B47 | B48 |
|  | 14 | 80 | 80 | 00 | 42 | 74 | 00 | 80 | 00 | 42 | 84 | 00 |
|  | B49 | B50 | B51 | B52 | B53 | B54 | B55 | B56 | B57 | B58 | B59 | B60 |
|  | 40 | 00 | 42 | 94 | 00 | 80 | 00 | 42 | 04 | 10 | 40 | 00 |

### 7.2.6	UE recognizing the priority order of the User controlled PLMN selector list using an ACT preference - UTRAN/E-UTRAN

Not applicable as UTRAN testing is involved.

### 7.2.7	UE recognizing the priority order of the User controlled PLMN selector list using an ACT preference - GSM/E-UTRAN

Not applicable as GERAN testing is involved.

### 7.2.8	UE recognising the priority order of the User controlled PLMN selector list with the same access technology – E-UTRAN in NB-S1 mode

#### 7.2.8.1	Definition and applicability

The User controlled PLMN selector list gives in priority order the preferred PLMNs of the User on which the UE shall register. The Radio Access Technology identifier defines the Radio network in which the UE shall register. The list is stored on the USIM in the EFPLMNwACT. Update and deletion of User controlled PLMNs may be performed by the subscriber by the use of the PIN.

#### 7.2.8.2	Conformance requirement

CR 1	When registering onto a VPLMN the UE shall consider the priority of the ACT identifier in the preferred list on the USIM.

Reference:

-	TS 22.011 [18], clause 3.2.2;

-	TS 31.102 [19], clauses 4.2.5 and 5.1.1.2.

CR 2	The modified value of EFEPSLOCI shall be available on the USIM when ending the test case.

#### 7.2.8.3	Test purpose

The purpose of this test is to verify that ACT with the higher priority (defined by its position in EFPLMNwACT) takes precedence over the UPLMN with the lower priority when the UE performs a network selection. Hereby the new coding for E-UTRAN in NB-S1 mode has to be handled correctly by the UE.

#### 7.2.8.4	Method of test

##### 7.2.8.4.1	Initial conditions

The values of the E-UTRAN/EPC ISIM UICC as defined in clause 4.5.5 of the present document are used with the following exception:

EFPLMNwACT (User Controlled PLMN Selector with Access Technology)

Logically:

1st PLMN:		244 083 (MCC MNC)

1st ACT:		E-UTRAN in NB-S1mode

2nd PLMN:	244 081

2nd ACT:		E-UTRAN in NB-S1 mode

3rd PLMN:	244 083

3rd ACT:		E-UTRAN

4th PLMN:	244 082

4th ACT:		GSM

5th PLMN:	244 003

5th ACT:		E-UTRAN

6th PLMN:	244 004

6th ACT:		UTRAN

7th PLMN:	244 005

7th ACT:		UTRAN

8th PLMN:	244 081

8th ACT:		UTRAN

9th PLMN:	244 007

9th ACT:		UTRAN

10th PLMN:	244 008

10th ACT:		E-UTRAN

11th PLMN:	244 009

11th ACT:		UTRAN

12th PLMN:	244 010

12th ACT:		E-UTRAN

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 42 | 34 | 80 | 50 | 00 | 42 | 14 | 80 | 50 | 00 | 42 | 34 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 80 | 40 | 00 | 42 | 24 | 80 | 00 | 80 | 42 | 34 | 00 | 40 |
|  | B25 | B26 | B27 | B28 | B29 | B30 | B31 | B32 | B33 | B34 | B35 | B36 |
|  | 00 | 42 | 44 | 00 | 80 | 00 | 42 | 54 | 00 | 80 | 00 | 42 |
|  | B37 | B38 | B39 | B40 | B41 | B42 | B43 | B44 | B45 | B46 | B47 | B48 |
|  | 14 | 80 | 80 | 00 | 42 | 74 | 00 | 80 | 00 | 42 | 84 | 00 |
|  | B49 | B50 | B51 | B52 | B53 | B54 | B55 | B56 | B57 | B58 | B59 | B60 |
|  | 40 | 00 | 42 | 94 | 00 | 80 | 00 | 42 | 04 | 10 | 40 | 00 |

The defined UICC/USIM configuration defined for this test case shall be used and made available on the UE. The UE is set to automatic PLMN selection mode.

The TT (NB-SS) transmits on the BCCH, with the following network parameters:

Cell A:

-	TAI (MCC/MNC/TAC):	244/081/0001.

-	Access control:	unrestricted.

Cell B:

-	TAI (MCC/MNC/TAC):	244/083/0001.

-	Access control:	unrestricted.

##### 7.2.8.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Run initial activation. |  |  |  |
| 2 | UE > TT | Send RRC CONNECTION REQUEST-NB | The RRC CONNECTION REQUEST-NB sent to the BCCH transmitting MCC/MNC 244/083 gets a RRC CONNECTION SETUP-NB as response from the TT | CR 1 |  |
| 3 | UE > TT | Send RRC CONNECTION SETUP COMPLETE-NB |  |  |  |
| 4 | UE > TT | Send ATTACH REQUEST | During registration the TT performs an authentication and starts integrity by using the security procedure. |  |  |
| 5 | TT > UE | Send ATTACH ACCEPT | The ATTACH ACCPT is sent with: - TAI (MCC/MNC/TAC): 244/083/ 0001 - GUTI: "24408300010266436587" | (CR 1) |  |
| 6 | UE > TT | Send ATTACH COMPLETE | The TT sends RRC CONNECTION RELEASE-NB |  |  |
| 7 | USER > UE | UE is soft powered down |  |  |  |
| 8 | TT | Verify the contents of EFEPSLOCI | Reading EFEPSLOCI may also be performed before step 7) | CR 2 | A.2/3 |

#### 7.2.8.5	Acceptance criteria

CR 1 is verified in step 2). The requirement is met if in step 2) the UE sends a RRC CONNECTION REQUEST-NB to the BCCH transmitting MCC/MNC 244/083 to the NB-SS

If the request is sent correctly can also be verified in the ATTACH ACCEPT send by the NB-SS if it is containing:

-	TAI (MCC/MNC/TAC):	244/083/ 0001

-	GUTI:	"24408300010266436587"

CR 2 is verified by reading the contents of EFEPSLOCI in step 8). CR 2 is met if the file contains the following data:

EFEPSLOCI (EPS Information)

Logically:

GUTI:							24408300010266436587

Last visited registered TAI:	244/083/0001

EPS update status:			updated

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 0B | F6 | 42 | 34 | 80 | 00 | 01 | 02 | 66 | 43 | 65 | 87 |
|  | B13 | B14 | B15 | B16 | B17 | B18 |
|  | 42 | 34 | 80 | 00 | 01 | 00 |

### 7.2.9	UE recognising the priority order of the User controlled PLMN selector list using the ACT preference – E-UTRAN in WB-S1/E-UTRAN in NB-S1

#### 7.2.9.1	Definition and applicability

The User controlled PLMN selector list gives in priority order the preferred PLMNs of the User on which the UE shall register. The Radio Access Technology identifier defines the Radio network in which the UE shall register. The list is stored on the USIM in the EFPLMNwACT. Update and deletion of User controlled PLMNs may be performed by the subscriber by the use of the PIN.

#### 7.2.9.2	Conformance requirement

CR 1	When registering onto a VPLMN the UE shall consider the priority of the ACT identifier in the preferred list on the USIM.

Reference:

-	TS 22.011 [18], clause 3.2.2;

-	TS 31.102 [19], clauses 4.2.5 and 5.1.1.2.

CR 2	The modified value of EFEPSLOCI shall be available on the USIM when ending the test case.

#### 7.2.9.3	Test purpose

The purpose of this test is to verify that ACT with the higher priority (defined by its position in EFPLMNwACT) takes precedence over the UPLMN with the lower priority when the UE performs a network selection. Hereby the new coding for RAT E-UTRAN in WB-S1 mode and E-UTRAN in NB-S1 mode has to be handled correctly by the UE.

#### 7.2.9.4	Method of test

##### 7.2.9.4.1	Initial conditions

The values of the E-UTRAN/EPC UICC as defined in clause 4.5.4 of the present document are used with the following exception:

EFPLMNwACT (User Controlled PLMN Selector with Access Technology)

Logically:

1st PLMN:		244 081 (MCC MNC)

1st ACT:		E-UTRAN

2nd PLMN:	244 081

2nd ACT:		GSM

3rd PLMN:	244 083

3rd ACT:		E-UTRAN in NB-S1 mode

4th PLMN:	244 083

4th ACT:		E-UTRAN in WB-S1 mode

5th PLMN:	244 003

5th ACT:		E-UTRAN

6th PLMN:	244 004

6th ACT:		UTRAN

7th PLMN:	244 005

7th ACT:		UTRAN

8th PLMN:	244 081

8th ACT:		UTRAN

9th PLMN:	244 007

9th ACT:		UTRAN

10th PLMN:	244 008

10th ACT:		E-UTRAN

11th PLMN:	244 009

11th ACT:	UTRAN

12th PLMN:	244 010

12th ACT:	E-UTRAN

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 42 | 14 | 80 | 40 | 00 | 42 | 14 | 80 | 00 | 80 | 42 | 34 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 80 | 50 | 00 | 42 | 34 | 80 | 60 | 00 | 42 | 34 | 00 | 40 |
|  | B25 | B26 | B27 | B28 | B29 | B30 | B31 | B32 | B33 | B34 | B35 | B36 |
|  | 00 | 42 | 44 | 00 | 80 | 00 | 42 | 54 | 00 | 80 | 00 | 42 |
|  | B37 | B38 | B39 | B40 | B41 | B42 | B43 | B44 | B45 | B46 | B47 | B48 |
|  | 14 | 80 | 80 | 00 | 42 | 74 | 00 | 80 | 00 | 42 | 84 | 00 |
|  | B49 | B50 | B51 | B52 | B53 | B54 | B55 | B56 | B57 | B58 | B59 | B60 |
|  | 40 | 00 | 42 | 94 | 00 | 80 | 00 | 42 | 04 | 10 | 40 | 00 |

The defined UICC/USIM configuration defined for this test case shall be used and made available on the UE. The UE is set to automatic PLMN selection mode.

The TT (E-USS and NB-SS) transmits on the BCCH, with the following network parameters on both cells:

-	TAI (MCC/MNC/TAC):	244/083/0001.

-	Access control:	unrestricted.

##### 7.2.9.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Run initial activation. |  |  |  |
| 2 | UE > TT | Send RRC CONNECTION REQUEST-NB | The RRC CONNECTION REQUEST-NB sent to the BCCH transmitting MCC/MNC 244/083 gets a RRC CONNECTION SETUP-NB as response from the TT | CR 1 |  |
| 3 | UE > TT | Send RRC CONNECTION SETUP COMPLETE-NB |  |  |  |
| 4 | UE > TT | Send ATTACH REQUEST | During registration the TT performs an authentication and starts integrity by using the security procedure. |  |  |
| 5 | TT > UE | Send ATTACH ACCEPT | The ATTACH ACCPT is sent with: - TAI (MCC/MNC/TAC): 244/083/ 0001 - GUTI: "24408300010266436587" | (CR 1) |  |
| 6 | UE > TT | Send ATTACH COMPLETE | The TT sends RRC CONNECTION RELEASE-NB |  |  |
| 7 | USER > UE | UE is soft powered down |  |  |  |
| 8 | TT | Verify the contents of EFEPSLOCI | Reading EFEPSLOCI may also be performed before step 7) | CR 2 | A.2/3 |

#### 7.2.9.5	Acceptance criteria

CR 1 is verified in step 2). The requirement is met if in step 2) the UE sends a RRC CONNECTION REQUEST-NB to the BCCH transmitting MCC/MNC 244/083 to the NB-SS

If the request is sent correctly can also be verified in the ATTACH ACCEPT send by the NB-SS if it is containing:

-	TAI (MCC/MNC/TAC):	244/083/ 0001

-	GUTI:	"24408300010266436587"

CR 2 is verified by reading the contents of EFEPSLOCI in step 8). CR 2 is met if the file contains the following data:

EFEPSLOCI (EPS Information)

Logically:

GUTI:							24408300010266436587

Last visited registered TAI:	244/083/0001

EPS update status:			updated

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 0B | F6 | 42 | 34 | 80 | 00 | 01 | 02 | 66 | 43 | 65 | 87 |
|  | B13 | B14 | B15 | B16 | B17 | B18 |
|  | 42 | 34 | 80 | 00 | 01 | 00 |

## 7.3	Operator controlled PLMN selector handling

### 7.3.1	UE recognizing the priority order of the Operator controlled PLMN selector list

GERAN/UTRAN test - not applicable

### 7.3.2	UE recognizing the priority order of the User controlled PLMN selector over the Operator controlled PLMN selector list

GERAN/UTRAN test - not applicable

### 7.3.3	UE recognizing the priority order of the Operator controlled PLMN selector list when accessing E-UTRAN

#### 7.3.3.1	Definition and applicability

The Operator controlled PLMN selector list gives in priority order the preferred OPLMNs on which the UE shall register if no network of the User controlled PLMN selector list is available. The Radio Access Technology identifier defines the Radio network in which the UE shall register. The list is stored on the USIM in the EFOPLMNwACT. Update and deletion of OPLMNs shall not be possible by the subscriber by the use of the PIN.

#### 7.3.3.2	Conformance requirement

CR 1	When registering onto a VPLMN the UE shall consider the priority of OPLMNs in the preferred list on the USIM.

Reference:

-	TS 22.011 [18], clause 3.2.2;

-	TS 31.102 [19], clause 4.2.53, 4.2.5 and 5.1.1.2.

CR 2	The modified value of EFEPSLOCI shall be available on the USIM when ending the test case.

#### 7.3.3.3	Test purpose

The purpose of this test is to verify that the OPLMN with the higher priority (defined by its position in EFOPLMNwACT) takes precedence over the OPLMN with the lower priority when the UE performs a network selection. Hereby the new coding for RAT E-UTRAN/NB-IoT has to be handled correctly by the UE.

#### 7.3.3.4	Method of test

##### 7.3.3.4.1	Initial conditions

The values of the E-UTRAN/EPC UICC as defined in clause 4.5.4 of the present document are used with the following exception:

EFUST (USIM Service Table)

Logically:

| Service n°42: |  | Operator controlled PLMN selector with Access Technology | available |
| --- | --- | --- | --- |
| Service n°86: |  | Allowed CSG Lists and corresponding indications | not available |

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary | xx1x xx11 | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxxx xx1x | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | B12 | B13 | B14 | B15 |
|  | xxxx xxxx | xxxx xxxx | xx01 xxxx | xxxx xxxx | xxxx xxxx | xxxx xxxx | xxxx xxxx |

The coding of EFUST shall conform with the capabilities of the USIM used.

EFOPLMNwACT (OPLMN Selector)

Logically:

1st PLMN:		254 012 (MCC MNC)

1st ACT		E-UTRAN

2nd PLMN:	254 011

2nd ACT		E-UTRAN

3rd PLMN:	254 002

3rd ACT:		E-UTRAN

4th PLMN:	254 012

4th ACT:		GSM

5th PLMN:	254 011

5th ACT:		GSM

6th PLMN:	254 005

6th ACT:		UTRAN

7th PLMN:	254 006

7th ACT:		UTRAN

8th PLMN:	254 007

8th ACT:		UTRAN

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 52 | 24 | 10 | 40 | 00 | 52 | 14 | 10 | 40 | 00 | 52 | 24 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 00 | 40 | 00 | 52 | 24 | 10 | 00 | 80 | 52 | 14 | 10 | 00 |
|  | B25 | B26 | B27 | B28 | B29 | B30 | B31 | B32 | B33 | B34 | B35 | B36 |
|  | 80 | 52 | 54 | 00 | 80 | 00 | 52 | 64 | 00 | 80 | 00 | 52 |
|  | B37 | B38 | B39 | B40 |
|  | 74 | 00 | 80 | 00 |

The defined UICC/USIM configuration defined for this test case shall be used and made available on the UE. The UE is set to automatic PLMN selection mode.

The TT (E-USS or NB-SS) transmits on the BCCH, with the following network parameters:

Cell A:

-	TAI (MCC/MNC/TAC):	254/011/0001.

-	Access control:			unrestricted.

Cell B:

-	TAI (MCC/MNC/TAC):	254/012/0001.

-	Access control:			unrestricted.

##### 7.3.3.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Run initial activation. |  |  |  |
| 2 | UE > TT | Send RRC CONNECTION REQUEST / RRC CONNECTION REQUEST-NB | The RRC CONNECTION REQUEST / RRC CONNECTION REQUEST-NB sent to the BCCH transmitting MCC/MNC 254/012 gets a RRC CONNECTION SETUP / RRC CONNECTION SETUP-NB as response from the TT | CR 1 |  |
| 3 | UE > TT | Send RRC CONNECTION SETUP COMPLETE / RRC CONNECTION SETUP COMPLETE-NB |  |  |  |
| 4 | UE > TT | Send ATTACH REQUEST | During registration the TT performs an authentication and starts integrity by using the security procedure. |  |  |
| 5 | TT > UE | Send ATTACH ACCEPT | The ATTACH ACCPT is sent with: - TAI (MCC/MNC/TAC): 254/012/0001 - GUTI: "25401200010266436587" | (CR 1) |  |
| 6 | UE > TT | Send ATTACH COMPLETE | The TT sends RRC CONNECTION RELEASE / RRC CONNECTION RELEASE-NB |  |  |
| 7 | USER > UE | UE is soft powered down |  |  |  |
| 8 | TT | Verify the contents of EFEPSLOCI | Reading EFEPSLOCI may also be performed before step 7) | CR 2 | A.2/3 |

#### 7.3.3.5	Acceptance criteria

CR 1 is verified in step 2). The requirement is met if in step 2) the UE sends a RRC CONNECTION REQUEST/
RRC CONNECTION REQUEST-NB to the BCCH transmitting MCC/MNC 254/012 to the E-USS/NB-SS

If the request is sent correctly can also be verified in the ATTACH ACCEPT send by the E-USS/NB-SS if it is containing:

-	TAI (MCC/MNC/TAC):	254/012/0001

-	GUTI:						"25401200010266436587"

CR 2 is verified by reading the contents of EFEPSLOCI in step 8). CR 2 is met if the file contains the following data:

EFEPSLOCI (EPS Information)

Logically:

GUTI:							25401200010266436587

Last visited registered TAI:	254/012/0001

EPS update status:			updated

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 0B | F6 | 52 | 24 | 10 | 00 | 01 | 02 | 66 | 43 | 65 | 87 |
|  | B13 | B14 | B15 | B16 | B17 | B18 |
|  | 52 | 24 | 10 | 00 | 01 | 00 |

### 7.3.4	UE recognizing the priority order of the User controlled PLMN selector over the Operator controlled PLMN selector list – E-UTRAN

#### 7.3.4.1	Definition and applicability

The User controlled PLMN selector list has a higher priority as the OPLMN selector list on which the UE shall register. The Radio Access Technology identifier defines the Radio network in which the UE shall register. The list is stored on the USIM in the EFPLMNwACT.

#### 7.3.4.2	Conformance requirement

CR 1	When registering onto a VPLMN the UE shall consider the priority of UPLMNs first before the OPLMNs in the preferred list on the USIM.

Reference:

-	TS 22.011 [18], clause 3.2.2;

-	TS 31.102 [19], clause 4.2.53, 4.2.5 and 5.1.1.2.

CR 2	The modified value of EFEPSLOCI shall be available on the USIM when ending the test case.

#### 7.3.4.3	Test purpose

The purpose of this test is to verify that the User controlled PLMN with a lower priority (defined by its position in EFPLMNwACT) takes precedence over the OPLMN with a higher priority when the UE performs a network selection. Hereby the new coding for RAT E-UTRAN/NB-IoT has to be handled correctly by the UE.

#### 7.3.4.4	Method of test

##### 7.3.4.4.1	Initial conditions

The values of the E-UTRAN/EPC UICC as defined in clause 4.5.4 of the present document are used with the following exception:

EFUST (USIM Service Table)

Logically:

| Service n°1: |  | Local Phone Book | available |
| --- | --- | --- | --- |
| Service n°2: |  | Fixed Dialling Numbers (FDN) | available |
| Service n°6: |  | Barred Dialling Numbers (BDN) | available |
| Service n°17: |  | Group Identifier Level 1 | not available |
| Service n°18: |  | Group Identifier Level 2 | not available |
| Service n°20: |  | User controlled PLMN selector with Access Technology | available |
| Service n°27: |  | GSM Access | available |
| Service n°33: |  | shall be set to '1' | available |
| Service n°34: |  | Enabled Services Table | available |
| Service n°42: |  | Operator controlled PLMN selector with Access Technology | available |
| Service n°85: |  | EPS Mobility Management Information | available |
| Service n°86: |  | Allowed CSG Lists and corresponding indications | not available |

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary | xx1x xx11 | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxxx xx1x | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | B12 | B13 | B14 | B15 |
|  | xxxx xxxx | xxxx xxxx | xx01 xxxx | xxxx xxxx | xxxx xxxx | xxxx xxxx | xxxx xxxx |

The coding of EFUST shall conform with the capabilities of the USIM used.

The defined UICC/USIM configuration defined for this test case shall be used and made available on the UE. The UE is set to automatic PLMN selection mode.

The TT (E-USS or NB-SS) transmits on the BCCH, with the following network parameters:

Cell A:

-	TAI (MCC/MNC/TAC):	254/011/0001.

-	Access control:			unrestricted.

Cell B:

-	TAI (MCC/MNC/TAC):	244/010/0001.

-	Access control:			unrestricted.

##### 7.3.4.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Run initial activation. |  |  |  |
| 2 | UE > TT | Send RRC CONNECTION REQUEST / RRC CONNECTION REQUEST-NB | The RRC CONNECTION REQUEST / RRC CONNECTION REQUEST-NB sent to the BCCH transmitting MCC/MNC 244/010 gets a RRC CONNECTION SETUP / RRC CONNECTION SETUP-NB as response from the TT | CR 1 |  |
| 3 | UE > TT | Send RRC CONNECTION SETUP COMPLETE / RRC CONNECTION SETUP COMPLETE-NB |  |  |  |
| 4 | UE > TT | Send ATTACH REQUEST | During registration the TT performs an authentication and starts integrity by using the security procedure. |  |  |
| 5 | TT > UE | Send ATTACH ACCEPT | The ATTACH ACCPT is sent with: - TAI (MCC/MNC/TAC): 244/010/0001 - GUTI: "24401000010266436587" | (CR 1) |  |
| 6 | UE > TT | Send ATTACH COMPLETE | The TT sends RRC CONNECTION RELEASE / RRC CONNECTION RELEASE-NB |  |  |
| 7 | USER > UE | UE is soft powered down |  |  |  |
| 8 | TT | Verify the contents of EFEPSLOCI | Reading EFEPSLOCI may also be performed before step 7) | CR 2 | A.2/3 |

#### 7.3.4.5	Acceptance criteria

CR 1 is verified in step 2). The requirement is met if in step 2) the UE sends a RRC CONNECTION REQUEST/
RRC CONNECTION REQUEST-NB to the BCCH transmitting MCC/MNC 244/010 to the E-USS/NB-SS

If the request is sent correctly can also be verified in the ATTACH ACCEPT send by the E-USS/NB-SS if it is containing:

-	TAI (MCC/MNC/TAC):	244/010/0001

-	GUTI:						"24401000010266436587"

CR 2 is verified by reading the contents of EFEPSLOCI in step 8). CR 2 is met if the file contains the following data:

EFEPSLOCI (EPS Information)

Logically:

GUTI:							24401000010266436587

Last visited registered TAI:	244/010/0001

EPS update status:			updated

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 0B | F6 | 42 | 04 | 10 | 00 | 01 | 02 | 66 | 43 | 65 | 87 |
|  | B13 | B14 | B15 | B16 | B17 | B18 |
|  | 42 | 04 | 10 | 00 | 01 | 00 |

### 7.3.5	UE recognising the priority order of the Operator controlled PLMN selector list when accessing E-UTRAN in NB-S1 mode

#### 7.3.5.1	Definition and applicability

The Operator controlled PLMN selector list gives in priority order the preferred OPLMNs on which the UE shall register if no network of the User controlled PLMN selector list is available. The Radio Access Technology identifier defines the Radio network in which the UE shall register. The list is stored on the USIM in the EFOPLMNwACT. Update and deletion of OPLMNs shall not be possible by the subscriber by the use of the PIN.

#### 7.3.5.2	Conformance requirement

CR 1	When registering onto a VPLMN the UE shall consider the priority of OPLMNs in the preferred list on the USIM.

Reference:

-	TS 22.011 [18], clause 3.2.2;

-	TS 31.102 [19], clause 4.2.53, 4.2.5 and 5.1.1.2.

CR 2	The modified value of EFEPSLOCI shall be available on the USIM when ending the test case.

#### 7.3.5.3	Test purpose

The purpose of this test is to verify that the OPLMN with the higher priority (defined by its position in EFOPLMNwACT) takes precedence over the OPLMN with the lower priority when the UE performs a network selection. Hereby the new coding for RAT E-UTRAN in NB-IoT mode has to be handled correctly by the UE.

#### 7.3.5.4	Method of test

##### 7.3.5.4.1	Initial conditions

The values of the E-UTRAN/EPC UICC as defined in clause 4.5.4 of the present document are used with the following exception:

EFUST (USIM Service Table)

Logically:

| Service n°42: |  | Operator controlled PLMN selector with Access Technology | available |
| --- | --- | --- | --- |
| Service n°86: |  | Allowed CSG Lists and corresponding indications | not available |

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary | xx1x xx11 | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxxx xx1x | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | B12 | B13 | B14 | B15 |
|  | xxxx xxxx | xxxx xxxx | xx01 xxxx | xxxx xxxx | xxxx xxxx | xxxx xxxx | xxxx xxxx |

The coding of EFUST shall conform with the capabilities of the USIM used.

EFOPLMNwACT (OPLMN Selector)

Logically:

1st PLMN:		254 012 (MCC MNC)

1st ACT		E-UTRAN in NB-S1 mode

2nd PLMN:	254 011

2nd ACT		E-UTRAN in NB-S1 mode

3rd PLMN:	254 002

3rd ACT:		E-UTRAN

4th PLMN:	254 012

4th ACT:		GSM

5th PLMN:	254 011

5th ACT:		GSM

6th PLMN:	254 005

6th ACT:		UTRAN

7th PLMN:	254 006

7th ACT:		UTRAN

8th PLMN:	254 007

8th ACT:		UTRAN

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 52 | 24 | 10 | 50 | 00 | 52 | 14 | 10 | 50 | 00 | 52 | 24 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 00 | 40 | 00 | 52 | 24 | 10 | 00 | 80 | 52 | 14 | 10 | 00 |
|  | B25 | B26 | B27 | B28 | B29 | B30 | B31 | B32 | B33 | B34 | B35 | B36 |
|  | 80 | 52 | 54 | 00 | 80 | 00 | 52 | 64 | 00 | 80 | 00 | 52 |
|  | B37 | B38 | B39 | B40 |
|  | 74 | 00 | 80 | 00 |

The defined UICC/USIM configuration defined for this test case shall be used and made available on the UE. The UE is set to automatic PLMN selection mode.

The TT (NB-SS) transmits on the BCCH, with the following network parameters:

Cell A:

-	TAI (MCC/MNC/TAC):	254/011/0001.

-	Access control:			unrestricted.

Cell B:

-	TAI (MCC/MNC/TAC):	254/012/0001.

-	Access control:			unrestricted.

##### 7.3.5.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Run initial activation |  |  |  |
| 2 | UE > TT | Send RRC CONNECTION REQUEST-NB | The RRC CONNECTION REQUEST-NB sent to the BCCH transmitting MCC/MNC 254/012 gets a RRC CONNECTION SETUP-NB as response from the TT | CR 1 |  |
| 3 | UE > TT | Send RRC CONNECTION SETUP COMPLETE-NB |  |  |  |
| 4 | UE > TT | Send ATTACH REQUEST | During registration the TT performs an authentication and starts integrity by using the security procedure. |  |  |
| 5 | TT > UE | Send ATTACH ACCEPT | The ATTACH ACCPT is sent with: - TAI (MCC/MNC/TAC): 254/012/0001 - GUTI: "25401200010266436587" | (CR 1) |  |
| 6 | UE > TT | Send ATTACH COMPLETE | The TT sends RRC CONNECTION RELEASE-NB |  |  |
| 7 | USER > UE | UE is soft powered down |  |  |  |
| 8 | TT | Verify the contents of EFEPSLOCI | Reading EFEPSLOCI may also be performed before step 7) | CR 2 | A.2/3 |

#### 7.3.5.5	Acceptance criteria

CR 1 is verified in step 2). The requirement is met if in step 2) the UE sends a RRC CONNECTION REQUEST-NB to the BCCH transmitting MCC/MNC 254/012 to the TT (NB-SS).

If the request is sent correctly can also be verified in the ATTACH ACCEPT send by the NB-SS if it is containing:

-	TAI (MCC/MNC/TAC):	254/012/0001

-	GUTI:						"25401200010266436587"

CR 2 is verified by reading the contents of EFEPSLOCI in step 8). CR 2 is met if the file contains the following data:

EFEPSLOCI (EPS Information)

Logically:

GUTI:							25401200010266436587

Last visited registered TAI:	254/012/0001

EPS update status:			updated

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 0B | F6 | 52 | 24 | 10 | 00 | 01 | 02 | 66 | 43 | 65 | 87 |
|  | B13 | B14 | B15 | B16 | B17 | B18 |
|  | 52 | 24 | 10 | 00 | 01 | 00 |

### 7.3.6	UE recognising the priority order of the User controlled PLMN selector over the Operator controlled PLMN selector list – E-UTRAN in NB-S1 mode

#### 7.3.6.1	Definition and applicability

The User controlled PLMN selector list has a higher priority as the OPLMN selector list on which the UE shall register. The Radio Access Technology identifier defines the Radio network in which the UE shall register. The list is stored on the USIM in the EFPLMNwACT.

#### 7.3.6.2	Conformance requirement

CR 1	When registering onto a VPLMN the UE shall consider the priority of UPLMNs first before the OPLMNs in the preferred list on the USIM.

Reference:

-	TS 22.011 [18], clause 3.2.2.2;

-	TS 31.102 [19], clause 4.2.53, 4.2.5 and 5.1.1.2.

CR 2	The modified value of EFEPSLOCI shall be available on the USIM when ending the test case.

#### 7.3.6.3	Test purpose

The purpose of this test is to verify that the User controlled PLMN with a lower priority (defined by its position in EFPLMNwACT) takes precedence over the OPLMN with a higher priority when the UE performs a network selection. Hereby the new coding for RAT E-UTRAN in NB-IoT mode has to be handled correctly by the UE.

#### 7.3.6.4	Method of test

##### 7.3.6.4.1	Initial conditions

The values of the E-UTRAN/EPC UICC as defined in clause 4.5.4 of the present document are used with the following exception:

EFUST (USIM Service Table)

Logically:

| Service n°42: |  | Operator controlled PLMN selector with Access Technology | available |
| --- | --- | --- | --- |
| Service n°86: |  | Allowed CSG Lists and corresponding indications | not available |

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary | xx1x xx11 | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxxx xx1x | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | B12 | B13 | B14 | B15 |
|  | xxxx xxxx | xxxx xxxx | xx01 xxxx | xxxx xxxx | xxxx xxxx | xxxx xxxx | xxxx xxxx |

The coding of EFUST shall conform with the capabilities of the USIM used.

EFPLMNwACT (User Controlled PLMN Selector with Access Technology)

Logically:

1st PLMN:		244 081 (MCC MNC)

1st ACT:		E-UTRAN

2nd PLMN:	244 081

2nd ACT:		GSM

3rd PLMN:	244 083

3rd ACT:		E-UTRAN

4th PLMN:	244 082

4th ACT:		GSM

5th PLMN:	244 003

5th ACT:		E-UTRAN

6th PLMN:	244 004

6th ACT:		UTRAN

7th PLMN:	244 005

7th ACT:		UTRAN

8th PLMN:	244 081

8th ACT:		UTRAN

9th PLMN:	244 007

9th ACT:		UTRAN

10th PLMN:	244 008

10th ACT:		E-UTRAN

11th PLMN:	244 009

11th ACT:		UTRAN

12th PLMN:	244 010

12th ACT:		E-UTRAN in NB-S1 mode

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 42 | 14 | 80 | 40 | 00 | 42 | 14 | 80 | 00 | 80 | 42 | 34 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 80 | 40 | 00 | 42 | 24 | 80 | 00 | 80 | 42 | 34 | 00 | 40 |
|  | B25 | B26 | B27 | B28 | B29 | B30 | B31 | B32 | B33 | B34 | B35 | B36 |
|  | 00 | 42 | 44 | 00 | 80 | 00 | 42 | 54 | 00 | 80 | 00 | 42 |
|  | B37 | B38 | B39 | B40 | B41 | B42 | B43 | B44 | B45 | B46 | B47 | B48 |
|  | 14 | 80 | 80 | 00 | 42 | 74 | 00 | 80 | 00 | 42 | 84 | 00 |
|  | B49 | B50 | B51 | B52 | B53 | B54 | B55 | B56 | B57 | B58 | B59 | B60 |
|  | 40 | 00 | 42 | 94 | 00 | 80 | 00 | 42 | 04 | 10 | 50 | 00 |

EFOPLMNwACT (OPLMN Selector)

Logically:

1st PLMN:		254 001 (MCC MNC)

1st ACT		E-UTRAN in NB-S1 mode

2nd PLMN:	254 001

2nd ACT		GSM

3rd PLMN:	254 002

3rd ACT:		E-UTRAN

4th PLMN:	254 003

4th ACT:		E-UTRAN

5th PLMN:	254 004

5th ACT:		UTRAN

6th PLMN:	254 005

6th ACT:		UTRAN

7th PLMN:	254 006

7th ACT:		UTRAN

8th PLMN:	254 007

8th ACT:		UTRAN

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 52 | 14 | 00 | 50 | 00 | 52 | 14 | 00 | 00 | 80 | 52 | 24 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 00 | 40 | 00 | 52 | 34 | 00 | 40 | 00 | 52 | 44 | 00 | 80 |
|  | B25 | B26 | B27 | B28 | B29 | B30 | B31 | B32 | B33 | B34 | B35 | B36 |
|  | 00 | 52 | 54 | 00 | 80 | 00 | 52 | 64 | 00 | 80 | 00 | 52 |
|  | B37 | B38 | B39 | B40 |
|  | 74 | 00 | 80 | 00 |

The defined UICC/USIM configuration defined for this test case shall be used and made available on the UE. The UE is set to automatic PLMN selection mode.

The TT (NB-SS) transmits on the BCCH, with the following network parameters:

Cell A:

-	TAI (MCC/MNC/TAC):	254/001/0001.

-	Access control:			unrestricted.

Cell B:

-	TAI (MCC/MNC/TAC):	244/010/0001.

-	Access control:			unrestricted.

##### 7.3.6.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Run initial activation |  |  |  |
| 2 | UE > TT | Send RRC CONNECTION REQUEST-NB | The RRC CONNECTION REQUEST-NB sent to the BCCH transmitting MCC/MNC 244/010 gets a RRC CONNECTION SETUP-NB as response from the TT | CR 1 |  |
| 3 | UE > TT | Send RRC CONNECTION SETUP COMPLETE-NB |  |  |  |
| 4 | UE > TT | Send ATTACH REQUEST | During registration the TT performs an authentication and starts integrity by using the security procedure. |  |  |
| 5 | TT > UE | Send ATTACH ACCEPT | The ATTACH ACCPT is sent with: - TAI (MCC/MNC/TAC): 244/010/0001 - GUTI: "24401000010266436587" | (CR 1) |  |
| 6 | UE > TT | Send ATTACH COMPLETE | The TT sends RRC CONNECTION RELEASE-NB |  |  |
| 7 | USER > UE | UE is soft powered down |  |  |  |
| 8 | TT | Verify the contents of EFEPSLOCI | Reading EFEPSLOCI may also be performed before step 7) | CR 2 | A.2/3 |

#### 7.3.6.5	Acceptance criteria

CR 1 is verified in step 2). The requirement is met if in step 2) the UE sends a RRC CONNECTION REQUEST-NB to the BCCH transmitting MCC/MNC 244/010 to the TT (NB-SS).

If the request is sent correctly can also be verified in the ATTACH ACCEPT send by the NB-SS if it is containing:

-	TAI (MCC/MNC/TAC):	244/010/0001

-	GUTI:						"24401000010266436587"

CR 2 is verified by reading the contents of EFEPSLOCI in step 8). CR 2 is met if the file contains the following data:

EFEPSLOCI (EPS Information)

Logically:

GUTI:							24401000010266436587

Last visited registered TAI:	244/010/0001

EPS update status:			updated

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 0B | F6 | 42 | 04 | 10 | 00 | 01 | 02 | 66 | 43 | 65 | 87 |
|  | B13 | B14 | B15 | B16 | B17 | B18 |
|  | 42 | 04 | 10 | 00 | 01 | 00 |

### 7.3.7	UE recognising the priority order of the Operator controlled PLMN selector list using the ACT preference – E-UTRAN in NB-S1/ E-UTRAN in WB-S1 mode

#### 7.3.7.1	Definition and applicability

The Operator controlled PLMN selector list gives in priority order the preferred OPLMNs on which the UE shall register if no network of the User controlled PLMN selector list is available. The Radio Access Technology identifier defines the Radio network in which the UE shall register. The list is stored on the USIM in the EFOPLMNwACT. Update and deletion of OPLMNs shall not be possible by the subscriber by the use of the PIN.

#### 7.3.7.2	Conformance requirement

CR 1	When registering onto a VPLMN the UE shall consider the priority of of OPLMNs in the preferred list on the USIM.

Reference:

-	TS 22.011 [18], clause 3.2.2;

-	TS 31.102 [19], clause 4.2.53, 4.2.5 and 5.1.1.2.

CR 2	The modified value of EFEPSLOCI shall be available on the USIM when ending the test case.

#### 7.3.7.3	Test purpose

The purpose of this test is to verify that the OPLMN with the higher priority (defined by its position in EFOPLMNwACT) takes precedence over the OPLMN with the lower priority when the UE performs a network selection. Hereby the new coding for RAT E-UTRAN in NB-IoT mode and E-UTRAN in WB-S1 mode has to be handled correctly by the UE.

#### 7.3.7.4	Method of test

##### 7.3.7.4.1	Initial conditions

The values of the E-UTRAN/EPC UICC as defined in clause 4.5.4 of the present document are used with the following exception:

EFUST (USIM Service Table)

Logically:

| Service n°42: |  | Operator controlled PLMN selector with Access Technology | available |
| --- | --- | --- | --- |
| Service n°86: |  | Allowed CSG Lists and corresponding indications | not available |

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary | xx1x xx11 | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxxx xx1x | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | B12 | B13 | B14 | B15 |
|  | xxxx xxxx | xxxx xxxx | xx01 xxxx | xxxx xxxx | xxxx xxxx | xxxx xxxx | xxxx xxxx |

The coding of EFUST shall conform with the capabilities of the USIM used.

EFOPLMNwACT (OPLMN Selector)

Logically:

1st PLMN:		254 012 (MCC MNC)

1st ACT		E-UTRAN in NB-S1 mode

2nd PLMN:	254 012

2nd ACT		E-UTRAN in WB-S1 mode

3rd PLMN:	254 002

3rd ACT:		E-UTRAN

4th PLMN:	254 012

4th ACT:		GSM

5th PLMN:	254 011

5th ACT:		GSM

6th PLMN:	254 005

6th ACT:		UTRAN

7th PLMN:	254 006

7th ACT:		UTRAN

8th PLMN:	254 007

8th ACT:		UTRAN

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 52 | 24 | 10 | 50 | 00 | 52 | 24 | 10 | 60 | 00 | 52 | 24 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 00 | 40 | 00 | 52 | 24 | 10 | 00 | 80 | 52 | 14 | 10 | 00 |
|  | B25 | B26 | B27 | B28 | B29 | B30 | B31 | B32 | B33 | B34 | B35 | B36 |
|  | 80 | 52 | 54 | 00 | 80 | 00 | 52 | 64 | 00 | 80 | 00 | 52 |
|  | B37 | B38 | B39 | B40 |
|  | 74 | 00 | 80 | 00 |

The defined UICC/USIM configuration defined for this test case shall be used and made available on the UE. The UE is set to automatic PLMN selection mode.

The TT (NB-SS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):	254/012/0001.

-	Access control:			unrestricted.

##### 7.3.7.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Run initial activation |  |  |  |
| 2 | UE > TT | Send RRC CONNECTION REQUEST-NB | The RRC CONNECTION REQUEST-NB sent to the BCCH transmitting MCC/MNC 254/012 gets a RRC CONNECTION SETUP-NB as response from the TT | CR 1 |  |
| 3 | UE > TT | Send RRC CONNECTION SETUP COMPLETE-NB |  |  |  |
| 4 | UE > TT | Send ATTACH REQUEST | During registration the TT performs an authentication and starts integrity by using the security procedure. |  |  |
| 5 | TT > UE | Send ATTACH ACCEPT | The ATTACH ACCPT is sent with: - TAI (MCC/MNC/TAC): 254/012/0001 - GUTI: "25401200010266436587" | (CR 1) |  |
| 6 | UE > TT | Send ATTACH COMPLETE | The TT sends RRC CONNECTION RELEASE-NB |  |  |
| 7 | USER > UE | UE is soft powered down |  |  |  |
| 8 | TT | Verify the contents of EFEPSLOCI | Reading EFEPSLOCI may also be performed before step 7) | CR 2 | A.2/3 |

#### 7.3.7.5	Acceptance criteria

CR 1 is verified in step 2). The requirement is met if in step 2) the UE sends a RRC CONNECTION REQUEST-NB to the BCCH transmitting MCC/MNC 254/012 to the TT (NB-SS).

If the request is sent correctly can also be verified in the ATTACH ACCEPT send by the NB-SS if it is containing:

-	TAI (MCC/MNC/TAC):	254/012/0001

-	GUTI:						"25401200010266436587"

CR 2 is verified by reading the contents of EFEPSLOCI in step 8). CR 2 is met if the file contains the following data:

EFEPSLOCI (EPS Information)

Logically:

GUTI:							25401200010266436587

Last visited registered TAI:	254/012/0001

EPS update status:			updated

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 0B | F6 | 52 | 24 | 10 | 00 | 01 | 02 | 66 | 43 | 65 | 87 |
|  | B13 | B14 | B15 | B16 | B17 | B18 |
|  | 52 | 24 | 10 | 00 | 01 | 00 |

### 7.3.8	UE recognising the priority order of the Operator controlled PLMN selector list using the ACT preference – E-UTRAN in NB-S1 mode/ GSM

Not applicable as GERAN testing is involved.

### 7.3.9	UE recognising the priority order of the Operator controlled PLMN selector list using the ACT preference – E-UTRAN in WB-S1 mode/GSM

Not applicable as GERAN testing is involved.

## 7.4	Higher priority PLMN search handling

### 7.4.1	UE recognizing the search period of the Higher priority PLMN

GERAN/UTRAN test - not applicable

### 7.4.2	GSM/UMTS dual mode UEs recognizing the search period of the Higher priority PLMN

GERAN/UTRAN test - not applicable

### 7.4.3	UE recognizing the search period of the Higher priority PLMN – E-UTRAN

#### 7.4.3.1	Definition and applicability

The Higher priority PLMN list gives in priority order the Higher priority PLMN on which the UE shall register first. The Higher priority PLMN search period gives the time interval in which the UE shall search for a possible Higher priority PLMN registration.

#### 7.3.7.2	Conformance requirement

CR 1	After registered onto a VPLMN the UE shall consider the Higher priority PLMN search period timer and the priority order of the Higher priority PLMNs in the preferred lists on the USIM.

Reference:

-	TS 22.011 [18], clause 3.2.2 and 3.2.2.5;

-	TS 24.301 [21], clause 5.5.3.2

-	TS 31.102 [19], clause 4.2.6.

CR 2	The modified value of EFEPSLOCI shall be available on the USIM when ending the test case.

#### 7.4.3.3	Test purpose

The purpose of this test is to verify that:

- the Higher priority PLMN timer is read

- the Higher priority PLMN takes precedence over the VPLMN in which the UE is currently registered in. Hereby the new coding for RAT E-UTRAN/NB-IoT has to be handled correctly by the UE.

#### 7.4.3.4	Method of test

##### 7.4.3.4.1	Initial conditions

The values of the E-UTRAN/EPC UICC as defined in clause 4.5.4 of the present document are used with the following exception:

EFHPPLMN (Higher Priority PLMN Search period)

Logically:

Time interval between two searches:	n minutes/n hours (2 hours)

Coding:

| Byte | B1 |
| --- | --- |
| Hex | 01 |

NOTE;	UEs only supporting any of the following or a combination of: NB-S1 mode or GERAN EC-GSM-IoT or Category M1 of E-UTRAN enhanced-MTC mode, T is 2 hours. Otherwise T is 6 minutes.

The defined UICC/USIM configuration defined for this test case shall be used and made available on the UE. The UE is set to automatic PLMN selection mode.

The TT (E-USS/NB-SS) transmits on the BCCH, with the following network parameters:

Cell A:

-	TAI (MCC/MNC/TAC):	244/008/0001.

-	Access control:			unrestricted.

Cell B, after the registration of UE:

-	TAI (MCC/MNC/TAC):	244/083/0001.

-	Access control:			unrestricted.

##### 7.4.3.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Run initial activation |  |  |  |
| 2 | UE > TT | Send RRC CONNECTION REQUEST / RRC CONNECTION REQUEST-NB | The RRC CONNECTION REQUEST / RRC CONNECTION REQUEST-NB sent to the BCCH transmitting MCC/MNC 244/008 gets a RRC CONNECTION SETUP / RRC CONNECTION SETUP-NB as response from the TT |  |  |
| 3 | UE > TT | Send RRC CONNECTION SETUP COMPLETE / RRC CONNECTION SETUP COMPLETE-NB |  |  |  |
| 4 | UE > TT | Send ATTACH REQUEST | During registration the TT performs an authentication and starts integrity by using the security procedure. |  |  |
| 5 | TT > UE | Send ATTACH ACCEPT | The ATTACH ACCPT is sent with: - TAI (MCC/MNC/TAC): 244/008/0001 - GUTI: "24400800010266436587" |  |  |
| 6 | UE > TT | Send ATTACH COMPLETE | The TT sends RRC CONNECTION RELEASE / RRC CONNECTION RELEASE-NB |  |  |
| 7 | TT | Start transmitting on Cell B | The E-USS/NB-SS starts to send on the second BCCH with the MCC/MNC 244/083. An internal timer shall start to run. |  |  |
| 8 | UE > TT | Send RRC CONNECTION REQUEST / RRC CONNECTION REQUEST-NB | The RRC CONNECTION REQUEST / RRC CONNECTION REQUEST-NB sent to the BCCH transmitting MCC/MNC 244/083 gets a RRC CONNECTION SETUP / RRC CONNECTION SETUP-NB as response from the TT | CR 1 |  |
| 9 | UE > TT | Send RRC CONNECTION SETUP COMPLETE / RRC CONNECTION SETUP COMPLETE-NB |  |  |  |
| 10 | UE > TT | Send TRACKING AREA UPDATE REQUEST | During registration the TT performs an authentication and starts integrity by using the security procedure. |  |  |
| 11 | TT > UE | Send TRACKING AREA UPDATE ACCEPT | The TRACKING AREA UPDATE ACCPT is sent with: - TAI (MCC/MNC/TAC): 244/083/0001 - GUTI: "24408300010266436587" | (CR 1) |  |
| 12 | UE > TT | Send TRACKING AREA UPDATE COMPLETE | The TT sends RRC CONNECTION RELEASE / RRC CONNECTION RELEASE-NB |  |  |
| 13 | USER > UE | UE is soft powered down |  |  |  |
| 14 | TT | Verify the contents of EFEPSLOCI | Reading EFEPSLOCI may also be performed before step 7) |  | A.2/3 |

#### 7.4.3.5	Acceptance criteria

CR 1 is verified in step 8). The requirement is met if in step 8) the UE sends a RRC CONNECTION REQUEST / RRC CONNECTION REQUEST-NB to the BCCH transmitting MCC/MNC 244/083 to the TT (E-USS or NB-SS).

If the request is sent correctly can also be verified in the TRACKING AREA UPDATE ACCEPT send by the TT (E-USS or NB-SS) if it is containing:

-	TAI (MCC/MNC/TAC):	244/083/0001

-	GUTI:						"24408300010266436587"

CR 2 is verified by reading the contents of EFEPSLOCI in step 14). CR 2 is met if the file contains the following data:

EFEPSLOCI (EPS Information)

Logically:

GUTI:							24408300010266436587

Last visited registered TAI:	244/083/0001

EPS update status:			updated

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 0B | F6 | 42 | 34 | 80 | 00 | 01 | 02 | 66 | 43 | 65 | 87 |
|  | B13 | B14 | B15 | B16 | B17 | B18 |
|  | 42 | 34 | 80 | 00 | 01 | 00 |

### 7.4.4	E-UTRAN/EPC capable UEs recognizing the search period of the Higher priority PLMN – GSM/E-UTRAN

Not applicable as GERAN testing is involved.

### 7.4.5	E-UTRAN/EPC capable UEs recognizing the search period of the Higher priority PLMN – UTRAN/E-UTRAN

Not applicable as UTRAN testing is involved.

### 7.4.6	UE recognising the search period of the higher priority PLMN – Satellite NG-RAN

FFS

### 7.4.7	UE recognising the search period of the higher priority PLMN – NG-RAN

#### 7.4.7.1	Definition and applicability

The higher priority PLMN list gives in priority order the higher priority PLMN on which the UE shall register first. The higher priority PLMN search period gives the time interval in which the UE shall search for a possible higher priority PLMN registration.

#### 7.4.7.2	Conformance requirement

CR 1	After registered onto a VPLMN the UE shall consider the higher priority PLMN search period timer and the priority order of the higher priority PLMNs in the preferred lists on the USIM.

Reference:

-	TS 22.011 [18], clauses 3.2.2 and 3.2.2.5;

-	TS 24.501 [25], clause 5.5.1.2.4;

-	TS 31.102 [19], clause 4.2.6.

CR 2	The UE shall start the search for a higher priority PLMN right after expiration of the search period timer.

Reference:

-	TS 23.122 [41], clause 4.4.3.3.1.1;

-	TS 31.102 [19], clause 4.2.6.

CR 3	The UE shall update EF5GS3GPPLOCI

Reference:

-	TS 24.501 [25], 9.11.3;

-	TS 31.102 [19], clause 4.4.11.2.

#### 7.4.7.3	Test purpose

To verify that the higher priority PLMN timer is read and the higher priority PLMN takes precedence over the VPLMN in which the UE is currently registered in. Hereby the new coding for NG-RAN has to be handled correctly by the UE.

#### 7.4.7.4	Method of test

##### 7.4.7.4.1	Initial conditions

The values of the 5G-NR UICC as defined in clause 4.5.9 of the present document are used with the following exceptions:

EFHPLMNwACT (HPLMN selector with Access Technology)

Logically:

1st PLMN:		244 083 (MCC MNC)

1st ACT:		NG-RAN

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 |
| --- | --- | --- | --- | --- | --- |
| Hex | 42 | 34 | 80 | 08 | 00 |

EFHPPLMN (Higher Priority HPLMN Search period)

Logically:

Time interval:	'01' - 6 minutes

Coding:

| Byte | B1 |
| --- | --- |
| Hex | 01 |

EFUST (USIM Service Table)

Logically:

| Service n°43: |  | HPLMN selector with access technology | available |
| --- | --- | --- | --- |
| Service n°127: |  | Control plane-based steering of UE in VPLMN | not available |

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary: | xxxx xx1x | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxxx x1xx | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | … | B16 |  |  |  |
|  | xxxx xxxx | xxxx xxxx | xxxx xx11 | ... | x0x0 111x |  |  |  |

EFPLMNwACT (User controlled PLMN Selector with Access Technology)

Logically:

1st PLMN:		244 081 (MCC MNC)

1st ACT:		E-UTRAN

2nd PLMN:	244 081

2nd ACT:		GSM

3rd PLMN:	244 083

3rd ACT:		NG-RAN

4th PLMN:	244 082

4th ACT:		GSM

5th PLMN:	244 003

5th ACT:		satellite NG-RAN

6th PLMN:	244 004

6th ACT:		UTRAN

7th PLMN:	244 005

7th ACT:		NG-RAN

8th PLMN:	244 081

8th ACT:		UTRAN

9th PLMN:	244 007

9th ACT:		UTRAN

10th PLMN:	244 008

10th ACT:		E-UTRAN

11th PLMN:	244 009

11th ACT:		UTRAN

12th PLMN:	244 010

12th ACT:		E-UTRAN

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 | B13 | B14 | B15 | B16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 42 | 14 | 80 | 40 | 00 | 42 | 14 | 80 | 00 | 80 | 42 | 34 | 80 | 08 | 00 | 42 |
|  | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 | B25 | B26 | B27 | B28 | B29 | B30 | B31 | B32 |
|  | 24 | 80 | 00 | 80 | 42 | 34 | 00 | 04 | 00 | 42 | 44 | 00 | 80 | 00 | 42 | 54 |
|  | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 | B41 | B42 | B43 | B44 | B45 | B46 | B47 | B48 |
|  | 00 | 08 | 00 | 42 | 14 | 80 | 80 | 00 | 42 | 74 | 00 | 80 | 00 | 42 | 84 | 00 |
|  | B49 | B50 | B51 | B52 | B53 | B54 | B55 | B56 | B57 | B58 | B59 | B60 |  |  |  |  |
|  | 40 | 00 | 42 | 94 | 00 | 80 | 00 | 42 | 04 | 10 | 40 | 00 |  |  |  |  |

The UICC/USIM configuration defined for this test case shall be used and made available on the UE. The UE is set to automatic PLMN selection mode.

The TT (NG-SS) is configured to transmit on the BCCH with the following network parameters:

Cell A:

-	TAI (MCC/MNC/TAC):		244/008/000001;

-	Access control:				unrestricted.

Cell B:

-	TAI (MCC/MNC/TAC):		244/083/000001;

-	Access control:				unrestricted.

The TT is transmitting on Cell A.

##### 7.4.7.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Run initial activation |  |  |  |
| 2 | UE > TT | Send RRCRequest | The UE sends a RRCRequest to the BCCH transmitting MCC/MNC 244/008. |  |  |
| 3 | TT > UE | Send RRCSetup | RRCSetup is sent from Cell A |  |  |
| 4 | UE > TT | Send RRCSetupComplete |  |  |  |
| 5 | UE > TT | Send REGISTRATION REQUEST | The TT initiates the authentication and starts integrity by using the security procedure. |  |  |
| 6 | TT > UE | Send REGISTRATION ACCEPT | The REGISTRATION ACCEPT is sent with: - TAI (MCC/MNC/TAC): 244/008/000001 - 5G-GUTI: "24400800010266436587" |  |  |
| 7 | UE > TT | Send REGISTRATION COMPLETE |  |  |  |
| 8 | TT > UE | Send RRCRelease |  |  |  |
| 9 | TT > UE | Start transmitting on Cell B | The TT (NG-SS) starts to send on the BCCH with the MCC/MNC 244/083. An internal timer shall start to run. |  |  |
| 10 | UE > TT | Send RRCRequest | The UE sends a RRCRequest to the BCCH transmitting MCC/MNC 244/083. The internal timer shall be stopped. | CR 2 |  |
| 11 | TT > UE | Send RRCSetup | RRCSetup is sent from Cell B |  |  |
| 12 | UE > TT | Send RRCSetupComplete |  |  |  |
| 13 | UE > TT | Send REGISTRATION REQUEST | The TT initiates the authentication and starts integrity by using the security procedure. |  |  |
| 14 | TT > UE | Send REGISTRATION ACCEPT | The REGISTRATION ACCEPT is sent with: - TAI (MCC/MNC/TAC): 244/083/000001 - 5G-GUTI: "24408300010266436587" |  |  |
| 15 | UE > TT | Send REGISTRATION COMPLETE |  | CR 1 |  |
| 16 | UE | Power off/deactivate the UE |  |  |  |
| 17 | TT | Verify the contents of EF5GS3GPPLOCI | Reading EF5GS3GPPLOCI may also be performed before step 16) | CR 3 | A.2/3 |

#### 7.4.7.5	Acceptance criteria

CR 1 is implicitly verified in step 15). The REGISTRATION COMPLETE on Cell B proves that the UE correctly handles the HPLMNwACT list.

CR 2 is verified in step 10), the UE shall send the RRCRequest to the higher priority PLMN after the given time interval is elapsed (+10%).

CR 3 is verified by reading the contents of EF5GS3GPPLOCI in step 17)

EF5GS3GPPLOCI (5GS 3GPP location information)

Logically:

5G-GUTI:

MCC:				244

MNC:				083

AMF Region ID:	0

AMF Set ID: 		4

AMF Pointer: 	2

5G-TMSI:			66 43 65 87

Last visited registered TAI in 5GS for 3GPP access:	244 083 000001

5GS update status for 3GPP access:					5U1 UPDATED

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 00 | 0B | F2 | 42 | 34 | 80 | 00 | 01 | 02 | 66 | 43 | 65 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |  |  |  |  |
|  | 87 | 42 | 34 | 80 | 00 | 00 | 01 | 00 |  |  |  |  |

## 7.5	Void

# 8	Subscription independent tests

## 8.1	Phone book procedures

### 8.1.1	Recognition of a previously changed phonebook

GERAN test - not applicable

### 8.1.2	Update of the Phonebook Synchronization Counter (PSC)

#### 8.1.2.1	Definition and applicability

The phonebook synchronisation counter is used to unambiguously identify the status of the phonebook. Every time the phonebook is reset/deleted or the UID and/or the CC has run out of range, the PSC shall be regenerated.

The PSC is a part of the phonebook identifier.

#### 8.1.2.2	Conformance requirement

CR 1	Every time either the UID or the CC is incremented by the ME, the value of the contend of the appropriate EF shall be tested.

Reference:

-	TS 31.102 [4], clause 4.4.2.12.2.

CR 2	If either UID or CC has reached "FF FF", the related EF shall be set to "00 01" and the PSC is incremented.

Reference:

-	TS 31.102 [4], clause 4.4.2.12.2.

#### 8.1.2.3	Test purpose

The purpose of this test is to verify that:

1)	the ME has recognised that the values of UID and CC has changed;

2)	the ME resets the value of EFUID and EFCC;

3)	the ME updates EFPSC.

#### 8.1.2.4	Method of test

##### 8.1.2.4.1	Initial conditions

The values of the Default UICC as defined in clause 4.5.2 of the present document are used with the following exceptions:

EFUID (Unique Identifier)

Logically:

one record is set to 'FF FF'

Coding:

| Byte | B1 | B2 |
| --- | --- | --- |
| Hex | FF | FF |

EFPUID (Previous Unique Identifier)

Logically:

record is set to 'FF FF'

Coding:

| Byte | B1 | B2 |
| --- | --- | --- |
| Hex | FF | FF |

EFCC (Change Counter)

Logically:

record is set to 'FF FF'

Coding:

| Byte | B1 | B2 |
| --- | --- | --- |
| Hex | FF | FF |

EFPSC (Phonebook Synchronisation Counter)

Logically:

record is set to '00 00 FF FF'

Coding:

| Byte | B1 | B2 | B3 | B4 |
| --- | --- | --- | --- | --- |
| Hex | 00 | 00 | FF | FF |

At least one phonebook entry shall be empty and available for creating a new entry (e.g. an appropriate ADN record).

The TT does not need to transmit on any BCCH.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and runs an initial activation.

##### 8.1.2.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | USER | Create a new phonebook entry | This may be done by storing a new telephone number in an empty ADN record |  |  |
| 2 | UE | Perform the regeneration of the UID records | It is assumed that the UE will indicate the time it needs to perform the regeneration by displaying a busy signal to the user. | (CR 1) |  |
| 3 | TT | READ EFUID, EFPSC and EFCC | Read and verify the content of the listed EFs | CR 1 CR 2 | A.2/1 OR A.2/2 OR A.2/3 |

#### 8.1.2.5	Acceptance criteria

CR 1 and CR 2 are explicitly verified in (or after) step 3) by any verification method listed in Table A.2. The requirements are met if:

- the EFUID (Unique Identifier) has been regenerated with UID values starting with "00 01".

NOTE:	The UID values may be stored in any order, but shall be unique. The entry in EFUID with value FF FF (the maximum value) shall have been replaced by an appropriate value which shall be distinguishable to the maximum value.

- EFPUID contains a UID value (other than FFFF) that is present in EFUID.

- EFCC contains the following value:

EFCC (Change Counter)

Logically:

record is set to "00 01"

Coding:

| Byte | B1 | B2 |
| --- | --- | --- |
| Hex | 00 | 01 |

- EFPSC contains the following value:

EFPSC (Phonebook Synchronisation Counter)

Logically:

record is set to "00 01 00 00"

Coding:

| Byte | B1 | B2 | B3 | B4 |
| --- | --- | --- | --- | --- |
| Hex | 00 | 01 | 00 | 00 |

### 8.1.3	Handling of BCD number/ SSC content extension

#### 8.1.3.1	Definition and applicability

The length of BCD number/SSC contents in EFADN byte gives the number of bytes of the following two data items containing actual BCD number/SSC information. This means that the maximum value is 11, even when the actual ADN/SSC information length is greater than 11. When an ADN/SSC has extension, it is indicated by the extension1 identifier being unequal to 'FF'. The remainder is stored in the EFEXT1 with the remaining length of the additional data being coded in the appropriate additional record itself.

#### 8.1.3.2	Conformance requirement

CR 1	The ME shall support the BCD number/SSC extension for EFADN.

Reference:

-	TS 31.102 [4], clauses 4.4.2.3 and 4.4.2.4.

#### 8.1.3.3	Test purpose

The purpose of this test is to verify that the ME is able to read and update BCD numbers/ SSC content with and without extension correctly in EFADN and EFEXT1.

#### 8.1.3.4	Method of test

##### 8.1.3.4.1	Initial conditions

The values of the Default UICC as defined in clause 4.5.2 of the present document are used with the following exceptions:

Only the global phonebook is present, containing an EFPBR (Phonebook Reference File) were only EFADN and EFEXT1 are present.

EFADN (Abbreviated dialling numbers)

Logically:

10 records, each record non-empty and unique. Unless otherwise stated, the ADN records shall not use extended BCD numbers/SSC strings

Record 1:

Length of alpha identifier:	32 characters;

Alpha identifier:				"Contact001";

Length of BCD number:		11 bytes;

TON and NPI:				Telephony and International;

Dialled number:				00112233445566778899;

CCI:							'FF';

Ext1:							'01'.

Coding for record 1:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 43 | 6F | 6E | 74 | 61 | 63 | 74 | 30 | 30 | 31 | FF | … |
|  | B32 | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 | B41 | B42 | B43 |
|  | FF | 0B | 91 | 00 | 11 | 22 | 33 | 44 | 55 | 66 | 77 | 88 |
|  | B44 | B45 | B46 |
|  | 99 | FF | 01 |

Record 2:

Length of alpha identifier:	32 characters;

Alpha identifier:				"Contact002";

Length of BCD number:		11 bytes;

TON and NPI:				Telephony and International;

Dialled number:				01234567890123456789;

CCI:							'FF';

Ext1:							'FF'.

Coding for record 2:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 43 | 6F | 6E | 74 | 61 | 63 | 74 | 30 | 30 | 32 | FF | … |
|  | B32 | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 | B41 | B42 | B43 |
|  | FF | 0B | 91 | 10 | 32 | 54 | 76 | 98 | 10 | 32 | 54 | 76 |
|  | B44 | B45 | B46 |
|  | 98 | FF | FF |

Record 3:

Length of alpha identifier:	32 characters;

Alpha identifier:				"Contact003";

Length of BCD number:		11 bytes;

TON and NPI:				Telephony and International;

Dialled number:				99887766554433221100;

CCI:							'FF';

Ext1:							'02'.

Coding for record 3:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 43 | 6F | 6E | 74 | 61 | 63 | 74 | 30 | 30 | 33 | FF | … |
|  | B32 | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 | B41 | B42 | B43 |
|  | FF | 0B | 91 | 99 | 88 | 77 | 66 | 55 | 44 | 33 | 22 | 11 |
|  | B44 | B45 | B46 |
|  | 00 | FF | 02 |

Record 4:

Length of alpha identifier:	32 characters;

Alpha identifier:				"Contact004";

Length of BCD number:		9 bytes;

TON and NPI:				Telephony and International;

Dialled number:				1212121212121212;

CCI:							'FF';

Ext1:							'FF'.

Coding for record 4:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 43 | 6F | 6E | 74 | 61 | 63 | 74 | 30 | 30 | 34 | FF | … |
|  | B32 | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 | B41 | B42 | B43 |
|  | FF | 09 | 91 | 21 | 21 | 21 | 21 | 21 | 21 | 21 | 21 | FF |
|  | B44 | B45 | B46 |
|  | FF | FF | 02 |

Record 7:

Length of alpha identifier:	32 characters;

Alpha identifier:				"Contact007";

Length of BCD number:		3 bytes;

TON and NPI:				Telephony and International;

Dialled number:				678;

CCI:							'FF';

Ext1:							'FF'.

Coding for record 7:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 43 | 6F | 6E | 74 | 61 | 63 | 74 | 30 | 30 | 37 | FF | … |
|  | B32 | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 | B41 | B42 | B43 |
|  | FF | 03 | 91 | 76 | F8 | FF | FF | FF | FF | FF | FF | FF |
|  | B44 | B45 | B46 |
|  | FF | FF | FF |

EFEXT1 (Extension 1)

Logically:

4 records

Record 1:

Record type:		'02';

Extension data:	"01234567890123456789";

Identifier:			'FF'.

Coding for record 1:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 02 | 0A | 10 | 32 | 54 | 76 | 98 | 10 | 32 | 54 | 76 | 98 |
|  | B13 |
|  | FF |

Record 2:

Record type:		'02';

Extension data:	"11p12345";

Identifier:			'FF'.

Coding for record 2:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 02 | 0A | 99 | 88 | 77 | 66 | 55 | 44 | 33 | 22 | 11 | 00 |
|  | B13 |
|  | 03 |

Record 3:

Record type:		'02';

Extension data:	"11p12345";

Identifier:			'FF'.

Coding for record 3:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 02 | 04 | 11 | 1C | 32 | 54 | FF | FF | FF | FF | FF | FF |
|  | B13 |
|  | FF |

Record 4:

Record type:		'00';

Extension data:	empty;

Identifier:			'FF'.

Coding for record 4:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 00 | FF | FF | FF | FF | FF | FF | FF | FF | FF | FF | FF |
|  | B13 |
|  | FF |

The TT does not need to transmit on any BCCH.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and runs an initial activation.

##### 8.1.3.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | USER | Select the global phonebook | The user shall use an MMI dependent procedure to select the global phonebook. |  |  |
| 2 | TT | READ EFPBR |  | CR 1 | A.2/1 OR A.2/2 |
| 3 | USER | Change BCD number of the entry "Contact002" from EFADN | The user shall change the BCD number of the entry "Contact002" to 22446622446622446600777888999. If the maximum number of BCD digits supported for the global phonebook update is less than in the requested input BCD number string, then the user shall enter the BCD number string as requested, but only up to the maximum number of BCD digits which are supported for updating. |  |  |
| 4 | TT | READ EFADN and EFEXT1 |  | CR 1 | A.2/1 OR A.2/2 (OR A.2/3) |
| 5 | USER | Extend the BCD number of the entry "Contact007" from EFADN | The user shall extend the BCD number of the entry "Contact007" to "01234567890123456789777888999". If the maximum number of BCD digits supported for the global phonebook update is less than in the requested input BCD number string, then the user shall enter the BCD number string as requested, but only up to the maximum number of BCD digits which are supported for updating. |  |  |
| 6 | TT | READ EFADN and EFEXT1 |  | CR 1 | A.2/1 OR A.2/2 (OR A.2/3) |
| 7 | USER | Delete the entry "Contact001" from EFADN |  |  |  |
| 8 | TT | READ EFADN and EFEXT1 |  | CR 1 | A.2/1 OR A.2/2 (OR A.2/3) |
| 9 | USER | Change BCD number of the entry "Contact002" from EFADN | The user shall change the BCD number of the entry "Contact002" to "22446622446600". |  |  |
| 10 | TT | READ EFADN and EFEXT1 |  | CR 1 | A.2/1 OR A.2/2 (OR A.2/3) |
| 11 | USER | Create a new entry in EFADN | The user shall create the new phonebook entry "NewContact" with the BCD number "1234567890123456789012345678901234567890123456789012". If the maximum number of BCD digits supported for the global phonebook update is less than in the requested input BCD number string, then the user shall enter the BCD number string as requested, but only up to the maximum number of BCD digits which are supported for updating. |  |  |
| 12 | TT | READ EFADN and EFEXT1 |  | CR 1 | A.2/1 OR A.2/2 (OR A.2/3) |
| 13 | USER | Delete the entry "Contact003" from EFADN |  |  |  |
| 14 | TT | READ EFADN and EFEXT1 |  | CR 1 | A.2/1 OR A.2/2 (OR A.2/3) |

#### 8.1.3.5	Acceptance criteria

CR 1 is explicitly verified in step 2) by using the methods A.2/1 or A.2/2 CR 1 the requirement is met if the ME has selected the global phonebook and has read EFPBR.

CR 1 is explicitly verified in (or after, when using method A.2/3) steps 4), 6), 8), 10), 12) and 14), by any verification method listed in Table A.2. The requirement is met if:

in step 4), the global phonebook contains a record with "22446622446622446600" as BCD number and "04" as extension record identifier. EFEXT1 shall contain a record with "Additional data" as record type, the BCD number extension "777888999" and "FF" as identifier to indicate the end of the chain.

NOTE:	If the maximum number of BCD digits supported for global phonebook updating is less than in the requested input BCD number, then EFADN and EFEXT1 shall contain the BCD number as entered on the MMI.has selected selected the global phonebook and shall have read EFPBR in the global phonebook.

in step 6), the ME acted to prevent storage of the extended BCD number, e.g. by giving an indication to the user or not allowing to enter the extended number. EFEXT1 has not been updated and the extension record identifier of the entry "Contact007" remains as "FF".

in step 8), the records of EFADN and EFEXT1 for the phonebook entry "Contact001" are empty, i.e. the EFADN record shall be "FF… FF" and the EFEXT1 record shall be "00FF… FF."

in step 10), the record of EFEXT1 which was used to store the BCD number extension "777888999" is empty and the record of EFADN used for storing the entry with the alpha identifier "Contact002" contains the BCD number "22446622446600" and the extension record identifier "FF".

in step 12), a record of EFADN contains "NewContact" as alpha identifier, "12345678901234567890" as BCD number and uses an extension record identifier unequal to "FF".

NOTE:	The EFEXT1 record which was indicated in the EFADN record used in this case shall contain "Additional data" as record type, "12345678901234567890" as BCD number and an extension record identifier unequal to "FF", while the EFEXT1 record used to continue the chain inside EFEXT1 shall contain "Additional data" as record type, "123456789012" as BCD number and "FF" as extension record identifier.
If the maximum number of BCD digits supported for global phonebook updating is less than the requested input BCD number, then EFADN and EFEXT1 shall contain the BCD number as entered on the MMI.

in step 14), the EFADN record, which was used to store the data for "Contact003" and the related records of EFEXT1 are empty.

If A.2/3 is the only method used, the reading of EFPBR in step 2) can be handled as implicitly verified if all the following phonebook actions can be executed successfully.

### 8.1.4	Phonebook selection

#### 8.1.4.1	Definition and applicability

The UICC may contain a global phonebook, or application specific phonebooks, or both in parallel. When both phonebook types co-exist, they are independent and no data is shared. In this case, it shall be possible for the user to select which phonebook the user would like to access.

#### 8.1.4.2	Conformance requirement

CR 1	The ME shall support the global and the application specific phonebooks.

Reference:

-	TS 31.102 [4], clauses 4.4.2.

#### 8.1.4.3	Test purpose

The purpose of this test is to verify that:

1)	the ME offers a possibility to select which phonebook the user would like to use. If both, the global and the local phonebook, co-exist;

2)	the data contained in the local phonebook can be read and updated correctly;

3)	the data contained in the global phonebook can be read and updated correctly.

#### 8.1.4.4	Method of test

##### 8.1.4.4.1	Initial conditions

The values of the Default UICC as defined in clause 4.5.2 of the present document are used with the following exceptions:

The local and the global phonebook are both present.

The local phonebook shall contain an:

EFPBR (Phonebook Reference File) were only EFADN and EFEXT1 are present:

EFADN (Abbreviated dialling numbers)

Logically:

10 records, each record non-empty and unique.

Record 4:

Length of alpha identifier:	32 characters;

Alpha identifier:				"Contact004";

Length of BCD number:		3 bytes;

TON and NPI:				Telephony and International;

Dialled number:				004;

CCI:							'FF';

Ext1:							'FF'.

Coding for record 4:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 43 | 6F | 6E | 74 | 61 | 63 | 74 | 30 | 30 | 34 | FF | … |
|  | B32 | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 | B41 | B42 | B43 |
|  | FF | 03 | 91 | 00 | F4 | FF | FF | FF | FF | FF | FF | FF |
|  | B44 | B45 | B46 |
|  | FF | FF | FF |

Record 5:

Length of alpha identifier:	32 characters;

Alpha identifier:				"Contact005";

Length of BCD number:		3 bytes;

TON and NPI:				Telephony and International;

Dialled number:				1234;

CCI:							'FF';

Ext1:							None.

Coding for record 5:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 43 | 6F | 6E | 74 | 61 | 63 | 74 | 30 | 30 | 35 | FF | … |
|  | B32 | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 | B41 | B42 | B43 |
|  | FF | 03 | 91 | 21 | 43 | FF | FF | FF | FF | FF | FF | FF |
|  | B44 | B45 | B46 |
|  | FF | FF | FF |

The global phonebook shall contain an:

EFPBR (Phonebook Reference File) were only EFADN is present:

EFADN (Abbreviated dialling numbers)

Logically:

8 records, record 3 and record 6 are empty, each record non-empty is unique.

Record 1:

Length of alpha identifier:	32 characters;

Alpha identifier:				"Contact001";

Length of BCD number:		3 bytes;

TON and NPI:				Telephony and International;

Dialled number:				001;

CCI:							'FF';

Ext1:							'FF'.

Coding for record 1:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 43 | 6F | 6E | 74 | 61 | 63 | 74 | 30 | 30 | 31 | FF | … |
|  | B32 | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 | B41 | B42 | B43 |
|  | FF | 03 | 91 | 00 | F1 | FF | FF | FF | FF | FF | FF | FF |
|  | B44 | B45 | B46 |
|  | FF | FF | FF |

Record 2:

Length of alpha identifier:	32 characters;

Alpha identifier:				"Contact002";

Length of BCD number:		3 bytes;

TON and NPI:				Telephony and International;

Dialled number:				002;

CCI:							'FF';

Ext1:							'FF'.

Coding for record 2:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 43 | 6F | 6E | 74 | 61 | 63 | 74 | 30 | 30 | 32 | FF | … |
|  | B32 | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 | B41 | B42 | B43 |
|  | FF | 03 | 91 | 00 | F2 | FF | FF | FF | FF | FF | FF | FF |
|  | B44 | B45 | B46 |
|  | FF | FF | FF |

Record 4:

Length of alpha identifier:	32 characters;

Alpha identifier:				"Contact004";

Length of BCD number:		3 bytes;

TON and NPI:				Telephony and International;

Dialled number:				0041;

CCI:							'FF';

Ext1:							'FF'.

Coding for record 4:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 43 | 6F | 6E | 74 | 61 | 63 | 74 | 30 | 30 | 34 | FF | … |
|  | B32 | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 | B41 | B42 | B43 |
|  | FF | 03 | 91 | 00 | 14 | FF | FF | FF | FF | FF | FF | FF |
|  | B44 | B45 | B46 |
|  | FF | FF | FF |

Record 5:

Length of alpha identifier:	32 characters;

Alpha identifier:				"Contact005";

Length of BCD number:		3 bytes;

TON and NPI:				Telephony and International;

Dialled number:				1234;

CCI:							'FF';

Ext1:							'FF'.

Coding for record 5:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 43 | 6F | 6E | 74 | 61 | 63 | 74 | 30 | 30 | 35 | FF | … |
|  | B32 | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 | B41 | B42 | B43 |
|  | FF | 03 | 91 | 21 | 43 | FF | FF | FF | FF | FF | FF | FF |
|  | B44 | B45 | B46 |
|  | FF | FF | FF |

Record 7:

Length of alpha identifier:	32 characters;

Alpha identifier:				"Contact007";

Length of BCD number:		3 bytes;

TON and NPI:				Telephony and International;

Dialled number:				007;

CCI:							'FF';

Ext1:							'FF'.

Coding for record 7:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 43 | 6F | 6E | 74 | 61 | 63 | 74 | 30 | 30 | 37 | FF | … |
|  | B32 | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 | B41 | B42 | B43 |
|  | FF | 03 | 91 | 00 | F7 | FF | FF | FF | FF | FF | FF | FF |
|  | B44 | B45 | B46 |
|  | FF | FF | FF |

Record 8:

Length of alpha identifier:	32 characters;

Alpha identifier:				"Contact008";

Length of BCD number:		3 bytes;

TON and NPI:				Telephony and International;

Dialled number:				008;

CCI:							'FF';

Ext1:							'FF'.

Coding for record 7:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 43 | 6F | 6E | 74 | 61 | 63 | 74 | 30 | 30 | 38 | FF | … |
|  | B32 | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 | B41 | B42 | B43 |
|  | FF | 03 | 91 | 00 | F8 | FF | FF | FF | FF | FF | FF | FF |
|  | B44 | B45 | B46 |
|  | FF | FF | FF |

The TT does not need to transmit on any BCCH.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and runs an initial activation.

##### 8.1.4.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | ME | READ EFUST | The ME shall have read the status of the local phonebook in EFUST | CR 1 | A.2/1 OR A.2/2 |
| 2 | USER | Select the global phonebook | The user shall use an MMI dependent procedure to select the global phonebook |  |  |
| 3 | TT | READ EFPBR | The ME shall have read EFPBR of the global phonebook. | CR 1 | A.2/1 OR A.2/2 |
| 4 | USER | Read "Contact005" | The global phonebook record with the alpha identifier "Contact005" and the dialling number "+1234" shall be presented to the user | CR 1 |  |
| 5 | USER | Change "Contact005" | The dialling number of the global phonebook record with the alpha identifier "Contact005" shall be set to "+1122330" |  |  |
| 6 | TT | READ EFADN | The ME shall have given an indication to the user that the phonebook update has been performed successfully | CR 1 | A.2/1 OR A.2/2 (OR A.2/3) |
| 7 | USER | Add an entry "Contact006" | The new entry with alpha identifier "Contact006" and "+9876543210" as associated dialling number shall be added to the global phonebook |  |  |
| 8 | TT | READ EFADN |  | CR 1 | A.2/1 OR A.2/2 (OR A.2/3) |
| 9 | USER | Select the local phonebook | The user shall use an MMI dependent procedure to select the local phonebook |  |  |
| 10 | TT | READ EFPBR | The ME shall have read EFPBR of the local phonebook. | CR 1 | A.2/1 OR A.2/2 |
| 11 | USER | Read "Contact005" | The local phonebook record with the alpha identifier "Contact005" and the dialling number "+1234" shall be presented to the user | CR 1 |  |
| 12 | USER | Change BCD number of the entry "Contact005" from EFADN | The user shall change the BCD number of the entry "Contact005" to "+11223345". |  |  |
| 13 | TT | READ EFADN | The ME shall have given an indication to the user that the phonebook update has been performed successfully | CR 1 | A.2/1 OR A.2/2 (OR A.2/3) |
| 14 | USER | Create a new entry in EFADN | The user shall try to add a new entry with the values "Contact007" as alpha identifier and "+007" as associated dialling number to the local phonebook |  |  |
| 15 | TT | READ EFADN | The ME shall have given an indication to the user that the phonebook update cannot be performed | CR 1 | A.2/1 OR A.2/2 (OR A.2/3) |
| 16 | USER | Delete the entry "Contact004" from EFADN |  |  |  |
| 17 | TT | READ EFADN | The ME shall have given an indication to the user that the deletion in the phonebook has been performed successfully | CR 1 | A.2/1 OR A.2/2 (OR A.2/3) |
| 18 | USER | Create a new entry in EFADN | The user shall try to add a new entry with the values "Contact007" as alpha identifier and "+007" as associated dialling number to the local phonebook |  |  |
| 19 | TT | READ EFADN |  | CR 1 | A.2/1 OR A.2/2 (OR A.2/3) |
| 20 | USER | Select the global phonebook | The user shall use an MMI dependent procedure to select the global phonebook |  |  |
| 21 | TT | READ EFPBR | The ME shall have read EFPBR of the global phonebook. | CR 1 | A.2/1 OR A.2/2 |
| 22 | USER | Delete the entry "Contact007" from EFADN |  |  |  |
| 23 | TT | READ EFADN | The ME shall have given an indication to the user that the deletion in the phonebook has been performed successfully | CR 1 | A.2/1 OR A.2/2 (OR A.2/3) |

#### 8.1.4.5	Acceptance criteria

In step 4), step 11) and step 15) CR 1 is verified via the MMI. The requirement is met if:

in steps 4) and 11), the correct phonebook entry is presented to the user;

in step 15) an appropriate error message is presented to the user.

CR 1 is explicitly verified in (or after, when using method A.2/3) steps 6), 8), 13), 17), 19) and 23), by any verification method listed in Table A.2. The requirement is met if:

in step 6), the user gets the indication that global phonebook update has been performed successfully, and if the global phonebook contains an ADN record with the alpha identifier "Contact005" and a dialling number set to "+1122330".

in step 8), the global phonebook contains an ADN record with the alpha identifier "Contact006" and a dialling number set to "+9876543210".

in step 13), the user gets the indication that the update of the local phonebook has been performed successfully, and if the local phonebook contains an ADN record with the alpha identifier "Contact005" and a dialling number set to "+11223345".

in step 17), the user gets the indication that the deletion from the local phonebook has been performed successfully, and if the record which was used to store the entry "Contact004" is empty.

in step 19), the local phonebook contains an ADN record with the alpha identifier "Contact007" and a dialling number set to "+007".

in step 23), the user gets the indication that the deletion from the global phonebook has been performed successfully, and if the record which was used to store the entry "Contact007" is empty.

If A.2/3 is the only method used, the reading of EFUST in step 1) and of EFPBR in steps 3), 10) and 21) can be handled as implicitly verified if all the following phonebook actions can be executed successfully.

### 8.1.5	Local Phonebook handling

#### 8.1.5.1	Definition and applicability

The UICC may contain a global phonebook, or application specific phonebooks, or both in parallel.

#### 8.1.5.2	Conformance requirement

CR 1	The ME shall support the the local phonebook.

Reference:

-	TS 31.102 [4], clauses 4.4.2.

#### 8.1.5.3	Test purpose

The purpose of this test is to verify that:

1)	the ME supports the local phonebook without existence of the global phonebook;

2)	the data contained in the local phonebook can be read and updated correctly;

#### 8.1.5.4	Method of test

##### 8.1.5.4.1	Initial conditions

The values of the Default UICC as defined in clause 4.5.2 of the present document are used with the following exceptions:

Only the local phonebook is present, containing an EFPBR (Phonebook Reference File) were only EFADN and EFEXT1 are present.

EFADN (Abbreviated dialling numbers)

Logically:

10 records, each record non-empty and unique.

Record 4:

Length of alpha identifier:	32 characters;

Alpha identifier:				"Contact004";

Length of BCD number:		3 bytes;

TON and NPI:				Telephony and International;

Dialled number:				004;

CCI:							'FF';

Ext1:							'FF'.

Coding for record 4:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 43 | 6F | 6E | 74 | 61 | 63 | 74 | 30 | 30 | 34 | FF | … |
|  | B32 | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 | B41 | B42 | B43 |
|  | FF | 03 | 91 | 00 | F4 | FF | FF | FF | FF | FF | FF | FF |
|  | B44 | B45 | B46 |
|  | FF | FF | FF |

Record 5:

Length of alpha identifier:	32 characters;

Alpha identifier:				"Contact005";

Length of BCD number:		3 bytes;

TON and NPI:				Telephony and International;

Dialled number:				1234;

CCI:							'FF';

Ext1:							None.

Coding for record 5:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 43 | 6F | 6E | 74 | 61 | 63 | 74 | 30 | 30 | 35 | FF | … |
|  | B32 | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 | B41 | B42 | B43 |
|  | FF | 03 | 91 | 21 | 43 | FF | FF | FF | FF | FF | FF | FF |
|  | B44 | B45 | B46 |
|  | FF | FF | FF |

The TT does not need to transmit on any BCCH.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and run an initial activation before step 1).

##### 8.1.5.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | USER | Select the global phonebook | The user shall use an MMI dependent procedure to select the global phonebook. |  |  |
| 2 | TT | READ EFPBR |  | CR 1 | A.2/1 OR A.2/2 |
| 3 | USER | Change BCD number of the entry "Contact002" from EFADN | The user shall change the BCD number of the entry "Contact002" to 22446622446622446600777888999. If the maximum number of BCD digits supported for the global phonebook update is less than in the requested input BCD number string, then the user shall enter the BCD number string as requested, but only up to the maximum number of BCD digits which are supported for updating. |  |  |
| 4 | TT | READ EFADN and EFEXT1 |  | CR 1 | A.2/1 OR A.2/2 (OR A.2/3) |
| 5 | USER | Extend the BCD number of the entry "Contact007" from EFADN | The user shall extend the BCD number of the entry "Contact007" to "01234567890123456789777888999". If the maximum number of BCD digits supported for the global phonebook update is less than in the requested input BCD number string, then the user shall enter the BCD number string as requested, but only up to the maximum number of BCD digits which are supported for updating. |  |  |
| 6 | TT | READ EFADN and EFEXT1 |  | CR 1 | A.2/1 OR A.2/2 (OR A.2/3) |
| 7 | USER | Delete the entry "Contact001" from EFADN |  |  |  |
| 8 | TT | READ EFADN and EFEXT1 |  | CR 1 | A.2/1 OR A.2/2 (OR A.2/3) |
| 9 | USER | Change BCD number of the entry "Contact002" from EFADN | The user shall change the BCD number of the entry "Contact002" to "22446622446600". |  |  |
| 10 | TT | READ EFADN and EFEXT1 |  | CR 1 | A.2/1 OR A.2/2 (OR A.2/3) |
| 11 | USER | Create a new entry in EFADN | The user shall create the new phonebook entry "NewContact" with the BCD number "1234567890123456789012345678901234567890123456789012". If the maximum number of BCD digits supported for the global phonebook update is less than in the requested input BCD number string, then the user shall enter the BCD number string as requested, but only up to the maximum number of BCD digits which are supported for updating. |  |  |
| 12 | TT | READ EFADN and EFEXT1 |  | CR 1 | A.2/1 OR A.2/2 (OR A.2/3) |
| 13 | USER | Delete the entry "Contact003" from EFADN |  |  |  |
| 14 | TT | READ EFADN and EFEXT1 |  | CR 1 | A.2/1 OR A.2/2 (OR A.2/3) |

#### 8.1.5.5	Acceptance criteria

CR 1 is explicitly verified in step 2) by using the methods A.2/1 or A.2/2 CR 1 the requirement is met if the ME has selected the global phonebook and has read EFPBR.

CR 1 is explicitly verified in (or after, when using method A.2/3) steps 4), 6), 8), 10), 12) and 14), by any verification method listed in Table A.2. The requirement is met if:

in step 4), the global phonebook contains a record with "22446622446622446600" as BCD number and "04" as extension record identifier. EFEXT1 shall contain a record with "Additional data" as record type, the BCD number extension "777888999" and "FF" as identifier to indicate the end of the chain.

NOTE:	If the maximum number of BCD digits supported for global phonebook updating is less than in the requested input BCD number, then EFADN and EFEXT1 shall contain the BCD number as entered on the MMI.has selected selected the global phonebook and shall have read EFPBR in the global phonebook.

in step 6), the ME acted to prevent storage of the extended BCD number, e.g. by giving an indication to the user or not allowing to enter the extended number. EFEXT1 has not been updated and the extension record identifier of the entry "Contact007" remains as "FF".

in step 8), the records of EFADN and EFEXT1 for the phonebook entry "Contact001" are empty, i.e. the EFADN record shall be "FF… FF" and the EFEXT1 record shall be "00FF… FF."

in step 10), the record of EFEXT1 which was used to store the BCD number extension "777888999" is empty and the record of EFADN used for storing the entry with the alpha identifier "Contact002" contains the BCD number "22446622446600" and the extension record identifier "FF".

in step 12), a record of EFADN contains "NewContact" as alpha identifier, "12345678901234567890" as BCD number and uses an extension record identifier unequal to "FF".

NOTE:	The EFEXT1 record which was indicated in the EFADN record used in this case shall contain "Additional data" as record type, "12345678901234567890" as BCD number and an extension record identifier unequal to "FF", while the EFEXT1 record used to continue the chain inside EFEXT1 shall contain "Additional data" as record type, "123456789012" as BCD number and "FF" as extension record identifier.
If the maximum number of BCD digits supported for global phonebook updating is less than the requested input BCD number, then EFADN and EFEXT1 shall contain the BCD number as entered on the MMI.

in step 14), the EFADN record, which was used to store the data for "Contact003" and the related records of EFEXT1 are empty.

If A.2/3 is the only method used, the reading of EFPBR in step 2) can be handled as implicitly verified if all the following phonebook actions can be executed successfully.

## 8.2	Short message handling report

### 8.2.1	Correct storage of a SM on the USIM

GERAN/UTRAN test - not applicable

### 8.2.2	Correct reading of a SM on the USIM

GERAN/UTRAN test - not applicable

### 8.2.3	SM memory capacity exceeded handling

GERAN/UTRAN test - not applicable

### 8.2.4A	Correct storage of an SM on the UICC

GERAN/UTRAN test - not applicable

### 8.2.4B	Correct storage of an SM on the UICC

FFS

### 8.2.5	Correct reading of a SM on the USIM if USIM and ISIM are present

FFS

### 8.2.6	Correct reading of a SM on the ISIM if USIM and ISIM are present

FFS

### 8.2.7	Correct storage of an SM on the UICC

FFS

## 8.3	MMS related tests

### 8.3.0	General condition

Currently MMS related test cases defined in TS 31.121 [2] are not used for device certification by any of the certification organisations. As long as there is no demand the tests from this clause will not be defined and set to FFS.

### 8.3.1	UE recognizing the priority order of MMS Issuer Connectivity Parameters

FFS

### 8.3.2	UE recognizing the priority order of MMS User Connectivity Parameters

FFS

### 8.3.3	UE recognizing the priority order of MMS Issuer Connectivity Parameters over the MMS User Connectivity Parameters

FFS

### 8.3.4	Usage of MMS notification

FFS

## 8.4	UICC presence detection

Not applicable for UEs with nrUSIMs.

## 8.5	UICC presence detection when connected to E-UTRAN/EPC

Not applicable for UEs with nrUSIMs.

# 9	USIM service handling

## 9.1	Access Point Name Control List handling

### 9.1.1	Access Point Name Control List handling for UEs supporting ACL

GERAN/UTRAN test - not applicable

### 9.1.2	Network provided APN handling for UEs supporting ACL

GERAN/UTRAN test - not applicable

### 9.1.3	Access Point Name Control List handling for UEs not supporting ACL

GERAN/UTRAN test - not applicable

### 9.1.4	Access Point Name Control List handling for UEs supporting ACL connected to E-UTRAN/EPC

#### 9.1.4.1	Definition and applicability

This EFACL contains the list of allowed APNs (Access Point Names). If the APN Control List service is enabled and the ME is to provide an APN as part of attach for PDN connectivity, then the ME shall verify that the APN value is present in the EFACL and if it is not the ME shall not proceed with the attach procedure. If the APN Control List service is enabled and the ME does not indend to provide an APN as part of the attach for PDN connectivity and use a network provided APN, the ME shall not check if "network provided APN" is contained within EFACL.

There is 1:1 mapping between one PDP context and one EPS Bearer.

Some MEs might not support the enablement or the disablement of the APN Control List service or the modification of the APN Control List. In these cases, the test sequence below provides separate branches.

#### 9.1.4.2	Conformance requirement

CR 1	The terminal shall support the APN Control List service as defined in TS 31.102 [19], clauses 5.1.1.2 and 5.3.14.

Reference:

-	TS 31.102 [19], clauses 4.2.8, 4.2.48, 5.1.1.2 and 5.3.14;

-	TS 23.060 [55], clause 9.2, 9.2.1A.

#### 9.1.4.3	Test purpose

The purpose of this test is to verify that:

1)	the ME consideres the status of the APN Control List service as indicated in EFUST and EFEST.

2)	The ME checks that the entire APN of any EPS Bearer is listed in EFACL before requesting this EPS Bearer activation from the network if the ACL service is enabled.

3)	the ME does not request the corresponding EPS Bearer activation from the network if the ACL service is enabled and the APN is not present in EFACL.

4)	the ME does not check that the network provided APN is present in EFACL during the initial attach procedure.

#### 9.1.4.4	Method of test

##### 9.1.4.4.1	Initial conditions

The values of the E-UTRAN/EPC UICC as defined in clause 4.5.4 of the present document are used with the following exceptions:

EFUST (USIM Service Table)

Logically:

| Service n°86: |  | Allowed CSG Lists and corresponding indications | not available |
| --- | --- | --- | --- |

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary | xx1x xx11 | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx x111 | xxxx xxxx | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | B12 | B13 | B14 | B15 |
|  | xxxx xxxx | xxxx xxxx | xx01 xxxx | xxxx xxxx | xxxx xxxx | xxxx xxxx | xxxx xxxx |

EFACL (Access Point Control List)

Logically:

Number of available bytes:	64

Number of APNs:			3

1st APN:					test.test

2nd APN:					3gpp.test

3rd APN:					2gpp.test

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 03 | DD | 0A | 04 | 74 | 65 | 73 | 74 | 04 | 74 | 65 | 73 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 74 | DD | 0A | 04 | 33 | 67 | 70 | 70 | 04 | 74 | 65 | 73 |
|  | B25 | B26 | B27 | B28 | B29 | B30 | B31 | B32 | B33 | B34 | B35 | B36 |
|  | 74 | DD | 0A | 04 | 32 | 67 | 70 | 70 | 04 | 74 | 65 | 73 |
|  | B37 | B38 | … | B64 |
|  | 74 | FF | … | FF |

The ME is configured to use the network provided APN for the initial attach procedure.

The TT (E-USS) transmits on the BCCH, with valid network parameters

##### 9.1.4.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Run initial activation |  |  |  |
| 2 | UE > TT | Attach to E-UTRAN | The ME shall have read the status of the ACL service in EFUST and EFEST. The ME shall be in updated idle mode on the E-USS. |  |  |
| 3 | USER > UE | Request PDN connectivity to "1gpp.test" | The ME shall not request PDN connectivity. | CR 1 |  |
| 4 | USER > UE | Request PDN connectivity to "3gpp.test" | PDN connectivity shall be activated. | CR 1 |  |
| 5 | USER > UE | Deactivate the PDN connectivity to "3gpp.test" | PDN connectivity shall be deactivated. | CR 1 |  |

| Steps to be executed if the user interface supports disableing the APN Control List service |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 6 | USER > UE | Disable the APN Control List service | When prompted to enter PIN2, the user shall present the correct PIN2 value (most probably 9999#). The APN Control List service shall have been set to disabled in EFEST. | CR 1 | A.2/3 |
| 7 | USER > UE | Request PDN connectivity to "1gpp.test" | PDN connectivity shall be activated. | CR 1 |  |
| 8 | USER > UE | Deactivate the PDN connectivity to "1gpp.test" | PDN connectivity shall be deactivated. | CR 1 |  |
| 9 | UE | The UE is powered off and then on |  |  |  |
| 10 | USER > UE | Enable the APN Control List service | When prompted to enter PIN2, the user shall present the correct PIN2 value (most probably 9999#). The APN Control List service shall have been set to enabled in EFEST. | CR 1 | A.2/3 |
| 11 | USER > UE | Request PDN connectivity to "1ppp.test" | The ME shall not request PDN connectivity. | CR 1 |  |
| 12 | UE | The UE is powered off and then on |  |  |  |
| 13 | USER > UE | Add the APN "1ppp.net" to the APN Control List | When prompted to enter PIN2, the user shall present the correct PIN2 value (most probably 9999#). APN "1ppp.net" shall have been added to the APN Control List in EFACL. | CR 1 | A.2/3 |
| 14 | USER > UE | Request PDN connectivity to "1ppp.test" | PDN connectivity shall be activated. | CR 1 |  |
| 15 | USER > UE | Deactivate the PDN connectivity | PDN connectivity shall be deactivated. | CR 1 |  |

| Common test steps for all UEs |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 16 | USER > UE | UE is soft powered down |  |  |  |

#### 9.1.4.5	Acceptance criteria

For devices not supporting a user interface supports to disable the APN Control List service CR 1 is implicitly verified at steps 3), 4) amd 5). The requirement is met if the UE is not requesting PDN connectivity in step 3), activating PDN connectivity in step 4) and deactivating PDN connectivity in step 5).

For devices supporting a user interface supports to disable the APN Control List service further aspects of CR 1 are verified in steps 6), 7), 8), 10), 11), 13), 14) and 15).

The requirements are met if the implicit verification in steps 7), 8), 11), 14) and 15) returns the expected results.

Additional verification is possible by exploring the contents of EFEST via method A.2/3 in steps 6), 10) and 13).

## 9.2	Service Dialling Numbers handling

### 9.2.1	Service Dialling Numbers handling

GERAN/UTRAN test - not applicable

# 10	CSG list handling

## 10.1	CSG list handling for E-UTRA

### 10.1.1	Automatic CSG selection in E-UTRA with CSG list on USIM, success

#### 10.1.1.1	Definition and applicability

A Closed Subscriber Group identifies subscribers of an operator who are permitted to access one or more cells of the PLMN but which have restricted access (CSG cells). A CSG cell is part of the PLMN, broadcasting a CSG indication that is set to TRUE and a specific CSG identity. A CSG cell is accessible by the members of the closed subscriber group for that CSG identity. For a CSG cell, the UE shall check the broadcast CSG ID against the Allowed CSG list provided by NAS to check whether a CSG cell is suitable for the UE.

#### 10.1.1.2	Conformance requirement

CR 1	The ME shall read the allowed CSG IDs from EFACSGL in order to perform HNB selection procedures. The lists in EFACSGL shall take precedence over the list stored in the ME non-volatile memory. If the MS supports CSG and if the list is available in the USIM, it is provisioned with a list of allowed CSG identities and associated PLMN identities from the USIM.

Reference:

-	TS 31.102 [19], clauses 4.4.6.2 and 5.8.1;

-	TS 23.122 [41], clause 3.1A.

CR 2	When a UE in idle mode detects the presence of a permissible CSG cell (a CSG cell whose CSG identity is in the UE's permitted CSG list), the UE shall select the CSG cell.

Reference:

-	TS 22.011 [18], clause 8.2.2.1.

#### 10.1.1.3	Test purpose

The purpose of this test is to verify that the ME only selects a CSG cell if the CSG IDs of the cell is found in the allowed CSG IDs from EFACSGL

#### 10.1.1.4	Method of test

##### 10.1.1.4.1	Initial conditions

The values of the Default UICC as defined in clause 4.5.2 of the present document are used with EFIMSI as defined in clause 4.6.2.

The TT (E-USS) transmits on the BCCH, with the following network parameters:

Cell A:

-	TAI (MCC/MNC/TAC):		246/081/0001.

-	Access control:				unrestricted.

-	csg-Indication:				TRUE

-	csg-Identity:					05

Cell B:

-	TAI (MCC/MNC/TAC):		246/081/0002.

-	Access control:				unrestricted.

-	csg-Indication:				TRUE

-	csg-Identity:					04

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and set the UEto automatic PLMN selection mode.

##### 10.1.1.4.2	Procedure

| Step | Direction | Action | Comment | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Run initial activation |  |  |  |
| 2 | UE | READ EFACSGL |  | CR 1 | A.2/1 OR A.2/2 |
| 3 | UE | The UE shall not attempt an Attach procedure, this is checked for 2 mins. |  | CR 2 |  |
| 4 | TT | TT stops all RF output for the first cell. |  |  |  |
| 5 | TT | The network parameters in BCCH of Cell A is reconfigured to indicate the following: - csg-Identity: 03 TT resumes RF output on the BCCH. |  |  |  |
| 6 | UE > TT | Send RRC CONNECTION REQUEST on first Cell (TAI 246/081/0001) | The TT responds with RRC CONNECTION SETUP |  |  |
| 7 | TT > UE | Send RRC CONNECTION SETUP COMPLETE |  | CR 2 |  |
| 8 | UE > TT | Send ATTACH REQUEST | The TT initiates authentication, starts integrity by using the security procedure. |  |  |
| 9 | TT > UE | Send ATTACH ACCEPT with: - TAI (MCC/MNC/TAC): 246/081/ 0001 - GUTI: "24608100010266436587" | UE sends ATTACH COMPLETE, and further TT sends RRC CONNECTION RELEASE |  |  |
| 10 | TT > UE | UE is soft powered down. |  |  |  |

#### 10.1.1.5	Acceptance criteria

CR 1 shall be explicitly verified in step 2) by ensuring that UE has successfully read EFACSGL

CR 2 shall be verified by ensuring that:

- UE has not attempted registration in step 3),

- UE has successfully registered to the permissible CSG cell that was pre-configured in EFACSGL at step 9). This may also be explicitly verified at step 9) by ensuring EFEPSLOCI contains the following data using any of the methods identified in clause 3.7.2 of the present document.

EFEPSLOCI (EPS Location Information)

Logically:

GUTI:								24608100010266436587

Last visited registered TAI:		246/081/0001

EPS update status:				updated

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 0B | F6 | 42 | 16 | 80 | 00 | 01 | 02 | 66 | 34 | 56 | 78 |
|  | B13 | B14 | B15 | B16 | B17 | B18 |
|  | 42 | 16 | 80 | 00 | 01 | 00 |

### 10.1.2	Automatic CSG selection in E-UTRA with CSG list on USIM, removal of CSG ID from the USIM

#### 10.1.2.1	Definition and applicability

A Closed Subscriber Group identifies subscribers of an operator who are permitted to access one or more cells of the PLMN but which have restricted access (CSG cells). A CSG cell is part of the PLMN, broadcasting a CSG indication that is set to TRUE and a specific CSG identity. A CSG cell is accessible by the members of the closed subscriber group for that CSG identity. For a CSG cell, the UE shall check the broadcast CSG ID against the Allowed CSG list provided by NAS to check whether a CSG cell is suitable for the UE.

#### 10.1.2.2	Conformance requirement

CR 1	If the UE receives ATTACH REJECT with cause #25 (Not authorized for this CSG) with integrity protection, the UE shall remove the CSG ID of the cell where the UE has sent the ATTACH REQUEST message from the Allowed CSG list.

Reference:

-	TS 22.011 [18], clause 8.2.2.1;

-	TS 31.102 [19], clauses 4.4.6.2 and 5.8.1;

-	TS 23.122 [41], clause 3.1A.

-	TS 24.301 [21], clause 5.5.1.2.5

#### 10.1.2.3	Test purpose

The purpose of this test is to verify that the ME removes the CSG ID from the Allowed CSG list in EFACSGL in case this CSG ID belongs to the cell where the ME has sent the ATTACH REQUEST message which was rejected with cause #25 by the E-USS.

#### 10.1.2.4	Method of test

##### 10.1.2.4.1	Initial conditions

The values of the Default UICC as defined in clause 4.5.2 of the present document are used with EFIMSI as defined in clause 4.6.2.

The TT (E-USS) transmits on the BCCH, with the following network parameters:

Cell A:

-	TAI (MCC/MNC/TAC):		246/081/0001.

-	Access control:				unrestricted.

-	csg-Indication:				TRUE

-	csg-Identity:					03

Cell B:

-	TAI (MCC/MNC/TAC):		246/081/0002.

-	Access control:				unrestricted.

-	csg-Indication:				TRUE

-	csg-Identity:					04

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and set the UE to automatic PLMN selection mode.

##### 10.1.2.4.2	Procedure

| Step | Direction | Action | Comment | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Run initial activation |  |  |  |
| 2 | UE > TT | Send RRC CONNECTION REQUEST on Cell A (TAI 246/081/0001) | The TT responds with RRC CONNECTION SETUP |  |  |
| 3 | TT > UE | Send RRC CONNECTION SETUP COMPLETE |  |  |  |
| 4 | UE > TT | Send ATTACH REQUEST | The TT initiates authentication, starts integrity by using the security procedure. |  |  |
| 5 | TT > UE | Send ATTACH REJECT with cause #25 (Not authorized for this CSG) with integrity protection. | TT sends RRC CONNECTION RELEASE |  |  |
| 6 | UE > USIM | UPDATE EFACSGL | This may be alternatively verified at step 8). | CR 1 | A.2/2 |
| 7 | TT > UE | UE is soft powered down. |  |  |  |

| Optional step if CR 1 verification cannot be performed at step 6) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 8 | UE | READ EFACSGL, EFEPSLOC |  | CR 1 | A.2/3 |

#### 10.1.2.5	Acceptance criteria

CR 1 shall be explicitly verified either in step 6) by analysing the UPDATE command used to update EFACSGL (A.2/1 or A.2/2) or at step 8) by reading the EFACSGL, EFEPSLOCI and verifying that is contains the following:

EFEPSLOCI (EPS Information)

Logically:

GUTI:							not checked

Last visited registered TAI:	246/081/0001

EPS update status:			ROAMING NOT ALLOWED

Coding;

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | xx | xx | xx | xx | xx | xx | xx | xx | xx | xx | xx | xx |
|  | B13 | B14 | B15 | B16 | B17 | B18 |
|  | 42 | 16 | 80 | 00 | 01 | 02 |

EFACSGL (Allowed CSG Lists)

Logically:

1st CSG list

PLMN:		246 081 (MCC MNC)

1st CSG list	1st CSG Type indication	02

1st CSG list	1st CSG HNB Name indication	02

1st CSG list	1st CSG CSG ID:	02 (27bit)

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | A0 | 0D | 80 | 03 | 42 | 16 | 80 | 81 | 06 | 02 | 02 | 00 |
|  | B13 | B14 | B15 |
|  | 00 | 00 | 5F |

2nd CSG list

PLMN:	244 081 (MCC MNC)

2nd CSG list	1st CSG Type indication	08

2nd CSG list	1st CSG HNB Name indication	08

2nd CSG list	1st CSG CSG ID:	08 (27bit)

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | A0 | 0D | 80 | 03 | 42 | 14 | 80 | 81 | 06 | 08 | 08 | 00 |
|  | B13 | B14 | B15 |
|  | 00 | 01 | 1F |

NOTE:	the 1st and 2nd CSG list may be stored together or separately in any record in arbitrary order.

### 10.1.3	Manual CSG selection in E-UTRA with CSG list on USIM, success

#### 10.1.3.1	Definition and applicability

A Closed Subscriber Group identifies subscribers of an operator who are permitted to access one or more cells of the PLMN but which have restricted access (CSG cells). A CSG cell is part of the PLMN, broadcasting a CSG indication that is set to TRUE and a specific CSG identity. A CSG cell is accessible by the members of the closed subscriber group for that CSG identity. For a CSG cell, the UE shall check the broadcast CSG ID against the Allowed CSG list provided by NAS to check whether a CSG cell is suitable for the UE.

A UE supporting CSG selection selects CSG cell either automatically based on the list of allowed CSG identities or manually based on user selection of CSG on indication of list of available CSGs.

#### 10.1.3.2	Conformance requirement

CR 1	In manual CSG selection mode, the ME indicates to the user the list of available CSGs in the currently selected PLMN. The list of CSGs presented to the user is not restricted by the allowed CSG list.
If the MS supports CSG, it is provisioned with a list of allowed CSG identities and associated PLMN identities from the USIM if the list is available in the USIM.

Reference:

-	TS 31.102 [19], clauses 4.4.6.2 and 5.8.1;

-	TS 23.122 [41], clause 3.1A.

CR 2	If the UE supporting CSG selection has attempted manual CSG selection, the UE, when receiving the TRACKING AREA UPDATE ACCEPT message, shall check if the CSG ID of the cell where the UE has sent the TRACKING AREA UPDATE REQUEST message is contained in the Allowed CSG list. If not, the UE shall add that CSG ID to the allowed CSG list.

Reference:

-	TS 24.301 [21], clause 5.5.3.2.4

#### 10.1.3.3	Test purpose

The purpose of this test is to verify that the ME adds the CSG ID to the Allowed CSG list in EFACSGL in case this CSG ID belongs to the cell where the ME has sent the TRACKING AREA UPDATE REQUEST message which was accepted by the E-USS.

#### 10.1.3.4	Method of test

##### 10.1.3.4.1	Initial conditions

The values of the Default UICC as defined in clause 4.5.2 of the present document are used with EFIMSI as defined in clause 4.6.2.

The TT (E-USS) transmits on the BCCH, with the following network parameters:

Cell A:

-	TAI (MCC/MNC/TAC):		246/081/0001.

-	Access control:		unrestricted.

-	csg-Indication:		FALSE

-	csg-Identity:		not present

Cell B:

-	TAI (MCC/MNC/TAC):		246/081/0002.

-	Access control:		unrestricted.

-	csg-Indication:		TRUE

-	csg-Identity:		04

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and set the UEto automatic PLMN selection mode.

##### 10.1.3.4.2	Procedure

| Step | Direction | Action | Comment | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Run initial activation |  |  |  |
| 2 | UE > TT | Send RRC CONNECTION REQUEST on Cell A (TAI 246/081/0001) | The TT responds with RRC CONNECTION SETUP |  |  |
| 3 | TT > UE | Send RRC CONNECTION SETUP COMPLETE |  |  |  |
| 4 | UE > TT | Send ATTACH REQUEST | The TT initiates authentication, starts integrity by using the security procedure. |  |  |
| 5 | TT > UE | Send ATTACH ACCEPT with: - TAI (MCC/MNC/TAC): 246/081/ 0001 - GUTI: "24608100010266345678" | UE sends ATTACH COMPLETE, and further TT sends RRC CONNECTION RELEASE |  |  |
| 6 | TT > UE | Use the MMI of the UE to perform manual CSG selection. | The UE shall indicate the availability of a cell with csg-Identity 04 for PLMN 246/081. The user shall select this cell by using the MMI. | CR 1 |  |
| 7 | UE > TT | Send RRC CONNECTION REQUEST on Cell A (TAI 246/081/0002) | The TT responds with RRC CONNECTION SETUP |  |  |
| 8 | TT > UE | Send RRC CONNECTION SETUP COMPLETE |  |  |  |
| 9 | UE > TT | Send TRACKING AREA UPDATE REQUEST | The TT initiates authentication, starts integrity by using the security procedure. |  |  |
| 10 | TT > UE | Send TRACKING AREA UPDATE ACCEPT with: - TAI (MCC/MNC/TAC): 246/081/ 0002 - GUTI: "24608100010266436599" | UE sends TRACKING AREA UPDATE COMPLETE, and further TT sends RRC CONNECTION RELEASE |  |  |
| 11 | UE > USIM | UPDATE EFACSGL | This may be alternatively verified at step 13). | CR 2 | A.2/2 |
| 12 | TT > UE | UE is soft powered down. |  |  |  |

| Optional step if CR 2 verification cannot be performed at step 11) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 13 | UE | READ EFACSGL |  | CR 2 | A.2/3 |

#### 10.1.3.5	Acceptance criteria

CR 1 is verified at step 6) by ensuring that during the manual CSG selection, the UE shall provide the information for a cell with csg-Identity 04 for PLMN 246/081 to the user.

CR 2 is verified explicitly at step 11) by ensuring that UE has successfully updated EFACSGL with CSG ID: 04 or at step 8) or step 13) by analysing the EFACSGL and verifying that is contains the following CSG lists:

EFEPSLOCI (EPS Information)

Logically:

GUTI:							24608100010266436599

Last visited registered TAI:	246/081/0002

EPS update status:			updated

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 0B | F6 | 42 | 16 | 80 | 00 | 01 | 02 | 66 | 43 | 65 | 99 |
|  | B13 | B14 | B15 | B16 | B17 | B18 |
|  | 42 | 16 | 80 | 00 | 02 | 00 |

EFACSGL (Allowed CSG Lists)

Logically:

1st CSG list

PLMN:		246 081 (MCC MNC)

1st CSG list	1st CSG Type indication	02

1st CSG list	1st CSG HNB Name indication	02

1st CSG list	1st CSG CSG ID:	02 (27bit)

1st CSG list	2nd CSG Type indication	03

1st CSG list	2nd CSG HNB Name indication	03

1st CSG list	2nd CSG CSG ID:	03 (27bit)

1st CSG list	3rd CSG Type indication	'xx' (not checked)

1st CSG list	3rd CSG HNB Name indication	'xx' (not checked)

1st CSG list	3rd CSG CSG ID:	04 (27bit)

Coding:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Coding: | A0 | 1D | 80 | 03 | 42 | 16 | 80 | 81 | 06 | 02 | 02 | 00 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 00 | 00 | 5F | 81 | 06 | 03 | 03 | 00 | 00 | 00 | 7F | 81 |
|  | B25 | B26 | B27 | B28 | B29 | B30 | B31 |
|  | 06 | xx | xx | 00 | 00 | 00 | 9F |

2nd CSG list

PLMN:		244 081 (MCC MNC)

2nd CSG list	1st CSG Type indication	08

2nd CSG list	1st CSG HNB Name indication	08

2nd CSG list	1st CSG CSG ID:	08 (27bit)

Coding:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Coding: | A0 | 0D | 80 | 03 | 42 | 14 | 80 | 81 | 06 | 08 | 08 | 00 |
|  | B13 | B14 | B15 |
|  | 00 | 01 | 1F |

NOTE:	The 1st and 2nd CSG list may be stored together or separately in any record in arbitrary order.
The CSG entries within each of the CSG lists are not sorted and may occur in arbitrary order.

### 10.1.4	Manual CSG selection in E-UTRA with CSG list on USIM, rejected

#### 10.1.4.1	Definition and applicability

A Closed Subscriber Group identifies subscribers of an operator who are permitted to access one or more cells of the PLMN but which have restricted access (CSG cells). A CSG cell is part of the PLMN, broadcasting a CSG indication that is set to TRUE and a specific CSG identity. A CSG cell is accessible by the members of the closed subscriber group for that CSG identity. For a CSG cell, the UE shall check the broadcast CSG ID against the Allowed CSG list provided by NAS to check whether a CSG cell is suitable for the UE.

A UE supporting CSG selection selects CSG cell either automatically based on the list of allowed CSG identities or manually based on user selection of CSG on indication of list of available CSGs.

#### 10.1.4.2	Conformance requirement

CR 1	If the UE supporting CSG selection has attempted manual CSG selection, the UE, when receiving the     TRACKING AREA UPDATE REJECT message with cause #25 (Not authorized for this CSG) with integrity protection, shall not add the CSG ID of the cell where the UE has sent the TRACKING AREA UPDATE REQUEST message if the CSG ID is contained in the Allowed CSG list.

Reference:

-	TS 31.102 [4], clauses 4.4.6.2 and 5.8.1;

-	TS 23.122 [31], clause 3.1A.

-	TS 24.301 [26], clause 5.5.3.2.5

#### 10.1.4.3	Test purpose

The purpose of this test is to verify that the ME does not add the CSG ID to the Allowed CSG list in EFACSGL in case this CSG ID belongs to the cell where the ME has sent the TRACKING AREA UPDATE REQUEST message which was rejected by the E-USS.

#### 10.1.4.4	Method of test

##### 10.1.4.4.1	Initial conditions

The values of the Default UICC as defined in clause 4.5.2 of the present document are used with EFIMSI as defined in clause 4.6.2.

The TT (E-USS) transmits on the BCCH, with the following network parameters:

Cell A:

-	TAI (MCC/MNC/TAC):		246/081/0001.

-	Access control:		unrestricted.

-	csg-Indication:		FALSE

-	csg-Identity:		not present

Cell B:

-	TAI (MCC/MNC/TAC):		246/081/0002.

-	Access control:		unrestricted.

-	csg-Indication:		TRUE

-	csg-Identity:				04

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and set the UEto automatic PLMN selection mode.

##### 10.1.4.4.2	Procedure

| Step | Direction | Action | Comment | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Run initial activation |  |  |  |
| 2 | UE > TT | Send RRC CONNECTION REQUEST on Cell A (TAI 246/081/0001) | The TT responds with RRC CONNECTION SETUP |  |  |
| 3 | TT > UE | Send RRC CONNECTION SETUP COMPLETE |  |  |  |
| 4 | UE > TT | Send ATTACH REQUEST | The TT initiates authentication, starts integrity by using the security procedure. |  |  |
| 5 | TT > UE | Send ATTACH ACCEPT with: - TAI (MCC/MNC/TAC): 246/081/ 0001 - GUTI: "24608100010266345678" | UE sends ATTACH COMPLETE, and further TT sends RRC CONNECTION RELEASE |  |  |
| 6 | TT > UE | Use the MMI of the UE to perform manual CSG selection | The UE shall indicate the availability of a cell with csg-Identity 04 for PLMN 246/081. The user shall select this cell by using the MMI. |  |  |
| 7 | UE > TT | Send RRC CONNECTION REQUEST on Cell A (TAI 246/081/0002) | The TT responds with RRC CONNECTION SETUP |  |  |
| 8 | TT > UE | Send RRC CONNECTION SETUP COMPLETE |  |  |  |
| 9 | UE > TT | Send TRACKING AREA UPDATE REQUEST | The TT initiates authentication, starts integrity by using the security procedure. |  |  |
| 10 | TT > UE | Send TRACKING AREA UPDATE REJECT with cause #25 (Not authorized for this CSG) with integrity protection. | TT sends RRC CONNECTION RELEASE |  |  |
| 11 | UE > USIM | Update EFACSGL | This step shall not occur. | CR 1 | A.2/2 |
| 12 | TT > UE | UE is soft powered down. |  |  |  |
| 13 | UE | Read EFACSGL |  | CR 1 | A.2/3 |

#### 10.1.4.5	Acceptance criteria

CR 1 shall be explicitly verified either at step 11) by ensuring that UE shall not update EFACSGL (A.2/1 or A.2/2) or alternatively at step 13) by analysing the EFACSGL, EFEPSLOCI and verifying that values shall be as follows:

EFEPSLOCI (EPS Information)

Logically:

GUTI:							not checked

Last visited registered TAI:	246/081/0001

EPS update status:			ROAMING NOT ALLOWED

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | xx | xx | xx | xx | xx | xx | xx | xx | xx | xx | xx | xx |
|  | B13 | B14 | B15 | B16 | B17 | B18 |
|  | 42 | 16 | 80 | 00 | 01 | 02 |

EFACSGL (Allowed CSG Lists)

Content not changed, shall be the same as defined in clause 4.5.4 of the present document.

### 10.1.5	CSG selection in E-UTRA with no CSG list on USIM, no IMSI change

#### 10.1.5.1	Definition and applicability

A Closed Subscriber Group identifies subscribers of an operator who are permitted to access one or more cells of the PLMN but which have restricted access (CSG cells). A CSG cell is part of the PLMN, broadcasting a CSG indication that is set to TRUE and a specific CSG identity. A CSG cell is accessible by the members of the closed subscriber group for that CSG identity. For a CSG cell, the UE shall check the broadcast CSG ID against the Allowed CSG list provided by NAS to check whether a CSG cell is suitable for the UE.

A UE supporting CSG selection selects CSG cell either automatically based on the list of allowed CSG identities or manually based on user selection of CSG on indication of list of available CSGs.

#### 10.1.5.2	Conformance requirement

CR 1	If the corresponding file is not present on the USIM, then the UE that supports CSG selection stores the allowed CSG list in a non-volatile memory in the ME together with the IMSI from the USIM. These EMM parameters can only be used if the IMSI from the USIM matches the IMSI stored in the non-volatile memory; else the UE shall delete the EMM parameters.

Reference:

-	TS 31.102 [19], clauses 4.4.6.2 and 5.8.1;

-	TS 24.301 [21], clause 5.5.3.2.4 and Annex C.

CR 2	To verify that the ME still has this CSG ID stored in the Allowed CSG list (in the non-volatile memory) available together with the IMSI after powered down and up in case the IMSI of the USIM has not changed.

Reference:

-	TS 31.102 [19], clauses 4.4.6.2 and 5.8.1;

-	TS 24.301 [21], clause 5.5.3.2.4 and Annex C.

CR 3	To verify that the ME removes the CSG ID from the Allowed CSG list inside the ME’s (non-volatile memory) in case this CSG ID belongs to the cell where the ME has sent the ATTACH REQUEST message which was rejected with cause #25 by the E-USS.

Reference:

-	TS 23.122 [41], clause 3.1A.

#### 10.1.5.3	Test purpose

The purpose of this test is to verify that:

- the ME adds the CSG ID to the Allowed CSG list in a non-volatile memory in the ME together with the IMSI from the USIM in case this CSG ID belongs to the cell where the ME has sent the TRACKING AREA UPDATE REQUEST message which was accepted by the E-USS as the corresponding file is not present on the simulated USIM.

- the ME still has this CSG ID stored in the Allowed CSG list available together with the IMSI after powered down and up in case the IMSI of the USIM has not changed.

- the ME removes the CSG ID from the Allowed CSG list inside the UE in case this CSG ID belongs to the cell where the ME has sent the ATTACH REQUEST message which was rejected with cause #25 by the EUSS.

#### 10.1.5.4	Method of test

##### 10.1.5.4.1	Initial conditions

The values of the Default UICC as defined in clause 4.5.2 of the present document are used with EFIMSI as defined in clause 4.6.2 and the following exceptions:

EFUST (USIM Service Table)

Logically:

| Service n°86: |  | Allowed CSG Lists and corresponding indications | not available |
| --- | --- | --- | --- |

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary | xx1x xx11 | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxxx xxxx | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | B12 |
|  | xxxx xxxx | xxxx xxxx | xx01 xxxx | 0000 0xxx |

The ME shall neither have stored csg-ID = 04 nor csg-ID=06 in the allowed CSG list in a non-volatile memory.

The TT (E-USS) transmits on the BCCH, with the following network parameters:

Cell A:

-	TAI (MCC/MNC/TAC):		246/081/0001.

-	Access control:			unrestricted.

-	csg-Indication:			FALSE

-	csg-Identity:			not present

Cell B:

-	TAI (MCC/MNC/TAC):		246/081/0002.

-	Access control:			unrestricted.

-	csg-Indication:			TRUE

-	csg-Identity:					04

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and set the UEto automatic PLMN selection mode.

##### 10.1.5.4.2	Procedure

| Step | Direction | Action | Comment | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Run initial activation |  |  |  |
| 2 | UE > TT | Send RRC CONNECTION REQUEST on Cell A (TAI 246/081/0001) | The TT responds with RRC CONNECTION SETUP |  |  |
| 3 | TT > UE | Send RRC CONNECTION SETUP COMPLETE |  |  |  |
| 4 | UE > TT | Send ATTACH REQUEST | The TT initiates authentication, starts integrity by using the security procedure. |  |  |
| 5 | TT > UE | Send ATTACH ACCEPT with: - TAI (MCC/MNC/TAC): 246/081/ 0001 - GUTI: "24608100010266345678" | UE sends ATTACH COMPLETE, and further TT sends RRC CONNECTION RELEASE |  |  |
| 6 | TT > UE | Use the MMI of the UE to perform manual CSG selection | The UE shall indicate the availability of a cell with csg-Identity 04 for PLMN 246/081. The user shall select this cell by using the MMI. |  |  |
| 7 | UE > TT | Send RRC CONNECTION REQUEST on csg cell (Cell B) (TAI 246/081/0002) | The TT responds with RRC CONNECTION SETUP |  |  |
| 8 | TT > UE | Send RRC CONNECTION SETUP COMPLETE |  |  |  |
| 9 | UE > TT | Send TRACKING AREA UPDATE REQUEST | The TT initiates authentication, starts integrity by using the security procedure. |  |  |
| 10 | TT > UE | Send TRACKING AREA UPDATE ACCEPT with the following parameters with integrity protection: - TAI (MCC/MNC/TAC): 246/081/0002 - GUTI: "24608100010266345699" | UE sends TRACKING AREA UPDATE COMPLETE, and further TT sends RRC CONNECTION RELEASE. |  |  |
| 11 | UE > USIM | UE is powered off and performs DETACH procedure |  |  |  |
| 12 | TT | The network parameters in BCCH of Cell A (246/081/0001) is reconfigured to indicate the following: - csg-Indication: TRUE - csg-Identity: 06 TT resumes RF output on the BCCH. |  |  |  |
| 13 | UE > USIM | UE is powered on | (with same default E-UTRAN UICC as defined in initial conditions) |  |  |
| 14 | UE > TT | Send RRC CONNECTION REQUEST on csg cell (Cell B) (TAI 246/081/0002) | The TT responds with RRC CONNECTION SETUP | CR 1CR 2 |  |
| 15 | TT > UE | Send RRC CONNECTION SETUP COMPLETE |  |  |  |
| 16 | UE > TT | Send ATTACH REQUEST | The TT initiates authentication, starts integrity by using the security procedure. |  |  |
| 17 | TT > UE | Send ATTACH REJECT with cause #25 (Not authorized for this CSG) with integrity protection. | TT sends RRC CONNECTION RELEASE |  |  |
| 18 | UE > USIM | UE is soft powered down |  |  |  |
| 19 | UE > USIM | UE is powered on (with same default E-UTRAN UICC as defined in initial conditions) |  |  |  |
| 20 | USER > UE | After 2 min, UE is powered down |  | CR 3 |  |

#### 10.1.5.5	Acceptance criteria

CR 1 and CR 2 are verified at step 14) by ensuring that UE shall send TRACKING AREA UPDATE REQUEST and complete the registration to the csg cell (TAI 246/081/0002 with csg identity: 02) successfully.

CR 3 is verified at step 20) by ensuring that the UE shall not attempt to register to the CSG cell (CSG ID: 04) where it previously (at step 17) received an ATTACH REJECT with cause #25.

### 10.1.6	CSG selection in E-UTRA with no CSG list on USIM, with IMSI change

#### 10.1.6.1	Definition and applicability

A Closed Subscriber Group identifies subscribers of an operator who are permitted to access one or more cells of the PLMN but which have restricted access (CSG cells). A CSG cell is part of the PLMN, broadcasting a CSG indication that is set to TRUE and a specific CSG identity. A CSG cell is accessible by the members of the closed subscriber group for that CSG identity. For a CSG cell, the UE shall check the broadcast CSG ID against the Allowed CSG list provided by NAS to check whether a CSG cell is suitable for the UE.

A UE supporting CSG selection selects CSG cell either automatically based on the list of allowed CSG identities or manually based on user selection of CSG on indication of list of available CSGs.

#### 10.1.6.2	Conformance requirement

CR 1	In manual CSG selection mode, the ME indicates to the user the list of available CSGs in the currently selected PLMN. The list of CSGs presented to the user is not restricted by the allowed CSG list.
If the MS supports CSG, it is provisioned with a list of allowed CSG identities and associated PLMN identities from the USIM if the list is available in the USIM. If the UE supporting CSG selection has attempted manual CSG selection, the UE, when receiving the TRACKING AREA UPDATE ACCEPT message, shall check if the CSG ID of the cell where the UE has sent the TRACKING AREA UPDATE REQUEST message is contained in the Allowed CSG list. If not, the UE shall add that CSG ID to the Allowed CSG list.

Reference:

-	TS 31.102 [19], clauses 4.4.6.2 and 5.8.1;

-	TS 23.122 [41], clause 3.1A.

-	TS 24.301 [21], clause 5.5.3.2.4 and Annex C.

CR 2	If the corresponding file is not present on the USIM, these EMM parameters except allowed CSG list are stored in a non-volatile memory in the ME together with the IMSI from the USIM. The allowed CSG list is stored in a non-volatile memory in the ME if the UE supports CSG selection. These EMM parameters can only be used if the IMSI from the USIM matches the IMSI stored in the non-volatile memory; else the UE shall delete the EMM parameters.

Reference:

-	TS 23.122 [41], clause 3.1A.

-	TS 24.301 [21], clause 5.5.3.2.4 and Annex C.

#### 10.1.6.3	Test purpose

The purpose of this test is to verify that:

- the ME adds the CSG ID to the Allowed CSG list in a non-volatile memory in the ME together with the IMSI from the USIM in case this CSG ID belongs to the cell where the ME has sent the TRACKING AREA UPDATE REQUEST message which was accepted by the E-USS as the corresponding file is not present on the configured USIM.

- the ME removes this CSG ID stored in the Allowed CSG list available together with the IMSI after powered down and up in case the IMSI of the USIM has changed.

#### 10.1.6.4	Method of test

##### 10.1.6.4.1	Initial conditions

The values of the Default UICC as defined in clause 4.5.2 of the present document are used with EFIMSI as defined in clause 4.6.2 and the following exceptions:

EFUST (USIM Service Table)

Logically:

| Service n°86: |  | Allowed CSG Lists and corresponding indications | not available |
| --- | --- | --- | --- |

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary | xx1x xx11 | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxxx xxxx | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | B12 |
|  | xxxx xxxx | xxxx xxxx | xx01 xxxx | 0000 0xxx |

The ME shall neither have stored csg-ID = 04 nor csg-ID=06 in the allowed CSG list in a non-volatile memory.

The TT (E-USS) transmits on the BCCH, with the following network parameters:

Cell A:

-	TAI (MCC/MNC/TAC):		246/081/0001.

-	Access control:		unrestricted.

-	csg-Indication:		FALSE

-	csg-Identity:		not present

Cell B:

-	TAI (MCC/MNC/TAC):		246/081/0002.

-	Access control:		unrestricted.

-	csg-Indication:		TRUE

-	csg-Identity:		04

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and set the UEto automatic PLMN selection mode.

##### 10.1.6.4.2	Procedure

| Step | Direction | Action | Comment | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Run initial activation |  |  |  |
| 2 | UE > TT | Send RRC CONNECTION REQUEST on the Cell A (TAI 246/081/0001) | The TT responds with RRC CONNECTION SETUP |  |  |
| 3 | TT > UE | Send RRC CONNECTION SETUP COMPLETE |  |  |  |
| 4 | UE > TT | Send ATTACH REQUEST | The TT initiates authentication, starts integrity by using the security procedure. |  |  |
| 5 | TT > UE | Send ATTACH ACCEPT with: - TAI (MCC/MNC/TAC): 246/081/ 0001 - GUTI: "24608100010266345678" | UE sends ATTACH COMPLETE, and further TT sends RRC CONNECTION RELEASE |  |  |
| 6 | TT > UE | Use the MMI of the UE to perform manual CSG selection | The UE shall indicate the availability of a cell with csg-Identity 04 for PLMN 246/081. The user shall select this cell by using the MMI. | CR 1 |  |
| 7 | UE > TT | Send RRC CONNECTION REQUEST on csg cell (Cell B) (TAI 246/081/0002) | The TT responds with RRC CONNECTION SETUP |  |  |
| 8 | TT > UE | Send RRC CONNECTION SETUP COMPLETE |  |  |  |
| 9 | UE > TT | Send TRACKING AREA UPDATE REQUEST | The TT initiates authentication, starts integrity by using the security procedure. |  |  |
| 10 | TT > UE | Send TRACKING AREA UPDATE ACCEPT with the following parameters with integrity protection: - TAI (MCC/MNC/TAC): 246/081/0002 - GUTI: "24608100010266345699" | UE sends TRACKING AREA UPDATE COMPLETE, and further TT sends RRC CONNECTION RELEASE. |  |  |
| 11 | UE > USIM | UE is powered off and performs DETACH procedure |  |  |  |
| 12 | UE > USIM | UE is powered on (with default UICC, with the following exception. The IMSI is set to "246081222233333".) |  |  |  |
| 13 | TT | The network parameters in BCCH of Cell A (246/081/0001) is reconfigured to indicate the following: - csg-Indication: TRUE - csg-Identity: 06 TT resumes RF output on the BCCH. |  |  |  |
| 14 | TT | Check if UE tries to register to the cell. | UE shall not try to register to Cell A. | CR 2 |  |
| 15 | USER > UE | After 2 min, UE is soft powered down |  |  |  |

#### 10.1.6.5	Acceptance criteria

CR 1	is verified at step 6) by ensuring that MMI of the UE shall indicate to the user the list of available CSGs in the currently selected PLMN, and user is able to select the CSG cell (CSG ID: 04)

CR 2	is verified at step 20) by ensuring that the UE shall not attempt to register to the CSG cell (CSG ID: 06).

### 10.1.7	Manual CSG selection without display restrictions in E-UTRA with ACSG list and OCSG list on USIM

#### 10.1.7.1	Definition and applicability

A Closed Subscriber Group identifies subscribers of an operator who are permitted to access one or more cells of the PLMN but which have restricted access (CSG cells). A CSG cell is part of the PLMN, broadcasting a CSG indication that is set to TRUE and a specific CSG identity. A CSG cell is accessible by the members of the closed subscriber group for that CSG identity. For a CSG cell, the UE shall check the broadcast CSG ID against the Allowed CSG list provided by NAS to check whether a CSG cell is suitable for the UE.

A UE supporting CSG selection selects a CSG cell either automatically based on the list of allowed CSG identities or manually based on user selection of CSG on indication of list of available CSGs.

Editor's note: it is still being investigated whether the testing of Automatic CSG selection should be performed using a separate test case or an enhanced version of the Manual CSG selection test case.

#### 10.1.7.2	Conformance requirement

CR 1	The ME shall read the allowed CSG IDs from EFACSGL and EFOCSGL in order to perform HNB selection procedures. The lists in EFACSGL and EFOCSGL shall take precedence over the list stored in the ME non-volatile memory.
If the MS supports CSG, it is provisioned with a list of allowed CSG identities and associated PLMN identities from the USIM if the list is available in the USIM.

Reference:

-	TS 31.102 [19], clauses 4.2.18, 4.4.6.2, 4.4.6.5 and 5.8.1;

-	TS 24.301 [21], clause 5.5.3.2.4

-	TS 22.220 [52], clause 5.3.2 and 5.5.4

CR 2	If the UE supporting CSG selection has attempted manual CSG selection, the UE, when receiving the TRACKING AREA UPDATE ACCEPT message, shall check if the CSG ID of the cell where the UE has sent the TRACKING AREA UPDATE REQUEST message is contained in the Allowed CSG list. If not, the UE shall add that CSG ID to the Allowed CSG list EFACSGL.

Reference:

-	TS 24.301 [21], clause 5.5.3.2.4

CR 3	By default, the UE shall display all available CSGs for any PLMN, unless the UE has been configured by the HPLMN, for a specific PLMN, to display only CSGs in the Operator CSG List that are available.

Reference:

-	TS 22.220 [52], clause 5.5.4

#### 10.1.7.3	Test purpose

The purpose of this test is to verify that:

- the ME adds the CSG ID to the Allowed CSG list in EFACSGL in case this CSG ID belongs to the cell where the ME has sent the TRACKING AREA UPDATE REQUEST message which was accepted by the E-USS.

- during the manual CSG selection all available CSG ID shall be displayed without restrictions.

#### 10.1.7.4	Method of test

##### 10.1.7.4.1	Initial conditions

The values of the ACSGL/OCSGL UICC as defined in clause 4.5.6 of the present document are used with EFIMSI as defined in clause 4.6.2.

The TT (E-USS) transmits on the BCCH, with the following network parameters:

Cell A:

-	TAI (MCC/MNC/TAC):	246/081/0001.

-	Access control:			unrestricted.

-	csg-Indication:			FALSE

-	csg-Identity:				not present

Cell B:

-	TAI (MCC/MNC/TAC):	246/081/0002.

-	Access control:			unrestricted.

-	csg-Indication:			TRUE

-	csg-Identity:				04

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and set the UEto automatic PLMN selection mode.

##### 10.1.7.4.2	Procedure

| Step | Direction | Action | Comment | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Run initial activation |  |  |  |
| 2 | UE > USIM | READ EFACSGL, EFOCSGL |  | CR 1 | A.2/1 OR A.2/2 |
| 3 | UE > TT | Send RRC CONNECTION REQUEST on Cell A (TAI 246/081/0001) | The TT responds with RRC CONNECTION SETUP |  |  |
| 4 | TT > UE | Send RRC CONNECTION SETUP COMPLETE |  |  |  |
| 5 | UE > TT | Send ATTACH REQUEST | The TT initiates authentication, starts integrity by using the security procedure. |  |  |
| 6 | TT > UE | Send ATTACH ACCEPT with: - TAI (MCC/MNC/TAC): 246/081/ 0001 - GUTI: "24608100010266345678" | UE sends ATTACH COMPLETE, and further TT sends RRC CONNECTION RELEASE |  |  |
| 7 | TT > UE | Use the MMI of the UE to perform manual CSG selection. | The UE shall indicate the availability of a cell with csg-Identity 04 for PLMN 246/081 (Cell B). The user shall select this cell by using the MMI. | CR 3 |  |
| 8 | UE > TT | Send RRC CONNECTION REQUEST on csg cell (Cell B) (TAI 246/081/0002) | The TT responds with RRC CONNECTION SETUP |  |  |
| 9 | TT > UE | Send RRC CONNECTION SETUP COMPLETE |  |  |  |
| 10 | UE > TT | Send TRACKING AREA UPDATE REQUEST | The TT initiates authentication, starts integrity by using the security procedure. |  |  |
| 11 | TT > UE | Send TRACKING AREA UPDATE ACCEPT with the following parameters with integrity protection: - TAI (MCC/MNC/TAC): 246/081/0002 - GUTI: "24608100010266345699" | UE sends TRACKING AREA UPDATE COMPLETE, and further TT sends RRC CONNECTION RELEASE. |  |  |
| 12 | UE > USIM | Update EFACSGL | In case the verification of the contents of EFACSGL is not done during the UPDATE command step 14) needs to be executed. | CR 2 | A.2/1 OR A.2/2 |
| 13 | UE > USIM | UE is soft powered down. |  |  |  |
| 14 | User > UE | READ EFEPSLOCI, EFACSGL, EFOCSGL |  |  | A.2/3 |

#### 10.1.7.5	Acceptance criteria

CR 1 shall be explicitly verified in step 2) by ensuring that UE has successfully read EFACSGL and EFOCSGL,

CR 3 is verified at step 7) by ensuring that MMI of the UE shall indicate to the user the list of available CSGs in the currently selected PLMN, and user is able to select the CSG cell (CSG ID: 04)

CR 2 shall be explicitly verified either in step 12) by analysing the UPDATE command used to update EFACSGL (A.2/1 or A.2/2) or at step 14) by reading the EFEPSLOCI, EFACSGL, EFOCSGL and verifying that is contains the following:

EFEPSLOCI (EPS Information)

Logically:

GUTI:							24608100010266436599

Last visited registered TAI:	246/081/0002

EPS update status:			updated

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 0B | F6 | 42 | 16 | 80 | 00 | 01 | 02 | 66 | 43 | 65 | 99 |
|  | B13 | B14 | B15 | B16 | B17 | B18 |
|  | 42 | 16 | 80 | 00 | 02 | 00 |

EFACSGL (Allowed CSG Lists)

Logically:

1st CSG list

PLMN:		246 081 (MCC MNC)

1st CSG list	1st CSG Type indication			02

1st CSG list	1st CSG HNB Name indication	02

1st CSG list	1st CSG CSG ID:					02 (27bit)

1st CSG list	2nd CSG Type indication			03

1st CSG list	2nd CSG HNB Name indication	03

1st CSG list	2nd CSG CSG ID:				03 (27bit)

1st CSG list	3rd CSG Type indication	'xx'		(not checked)

1st CSG list	3rd CSG HNB Name indication	'xx'	(not checked)

1st CSG list	3rd CSG CSG ID:					04 (27bit)

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | A0 | 1D | 80 | 03 | 42 | 16 | 80 | 81 | 06 | 02 | 02 | 00 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 00 | 00 | 5F | 81 | 06 | 03 | 03 | 00 | 00 | 00 | 7F | 81 |
|  | B25 | B26 | B27 | B28 | B30 | B31 | B32 |
|  | 06 | xx | xx | 00 | 00 | 00 | 9F |

2nd CSG list

PLMN:		244 081 (MCC MNC)

2nd CSG list	1st CSG Type indication			08

2nd CSG list	1st CSG HNB Name indication	08

2nd CSG list	1st CSG CSG ID:					08 (27bit)

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | A0 | 0D | 80 | 03 | 42 | 14 | 80 | 81 | 06 | 08 | 08 | 00 |
|  | B13 | B14 | B15 |
|  | 00 | 01 | 1F |

NOTE:	The 1st and 2nd CSG list may be stored together or separately in any record in arbitrary order.

EFOCSGL (Operator CSG Lists)

Unchanged, values as defined in clause 4.5.6 of the present document.

### 10.1.8	Manual CSG selection with display restrictions in E-UTRA with ACSG list and OCSG list on USIM

#### 10.1.8.1	Definition and applicability

A Closed Subscriber Group identifies subscribers of an operator who are permitted to access one or more cells of the PLMN but which have restricted access (CSG cells). A CSG cell is part of the PLMN, broadcasting a CSG indication that is set to TRUE and a specific CSG identity. A CSG cell is accessible by the members of the closed subscriber group for that CSG identity. For a CSG cell, the UE shall check the broadcast CSG ID against the Allowed CSG list provided by NAS to check whether a CSG cell is suitable for the UE.

A UE supporting CSG selection selects a CSG cell either automatically based on the list of allowed CSG identities or manually based on user selection of CSG on indication of list of available CSGs.

#### 10.1.8.2	Conformance requirement

CR 1	The ME shall read the allowed CSG IDs from EFACSGL and EFOCSGL in order to perform HNB selection procedures. The lists in EFACSGL and EFOCSGL shall take precedence over the list stored in the ME non-volatile memory.
If the MS supports CSG, it is provisioned with a list of allowed CSG identities and associated PLMN identities from the USIM if the list is available in the USIM.

Reference:

-	TS 31.102 [19], clauses 4.2.18, 4.4.6.2, 4.4.6.5 and 5.8.1;

-	TS 24.301 [21], clause 5.5.3.2.4

CR 2	If the UE supporting CSG selection has attempted manual CSG selection, the UE, when receiving the TRACKING AREA UPDATE ACCEPT message, shall check if the CSG ID of the cell where the UE has sent the TRACKING AREA UPDATE REQUEST message is contained in the Allowed CSG list. If not, the UE shall add that CSG ID to the Allowed CSG list EFACSGL.

Reference:

-	TS 24.301 [21], clause 5.5.3.2.4

CR 3	By default, the UE shall display all available CSGs for any PLMN, unless the UE has been configured by the HPLMN, for a specific PLMN, to display only CSGs in the Operator CSG List that are available.

Reference:

-	TS 22.220 [52], clause 5.3.2 and 5.5.4

#### 10.1.8.3	Test purpose

The purpose of this test is to verify that:

- the ME adds the CSG ID to the Allowed CSG list in EFACSGL in case this CSG ID belongs to the cell where the ME has sent the TRACKING AREA UPDATE REQUEST message which was accepted by the TT (E-USS).

- during the manual CSG selection all available CSG ID shall be displayed with restrictions.

#### 10.1.8.4	Method of test

##### 10.1.8.4.1	Initial conditions

The values of the ACSGL/OCSGL UICC as defined in clause 4.5.6 of the present document are used with EFIMSI as defined in clause 4.6.2 and the following exceptions:

EFAD (Administrative Data)

Logically:

UE operation mode:		Normal operation + specific facilities

Additional information:	Ciphering indicator feature disabled

MNC:						3 digit
For every PLMN not included in EF_OCSGL or any PLMN for which a CSG display indicator tag is not present, only the available CSGs found in the Operator CSG list shall be displayed (B3)

Coding:

| Byte | B1 | B2 | B3 | B4 |
| --- | --- | --- | --- | --- |
| Hex | 01 | 00 | 02 | 03 |

The TT (E-USS) transmits on the BCCH, with the following network parameters:

Cell A:

-	TAI (MCC/MNC/TAC):		246/081/0001.

-	Access control:		unrestricted.

-	csg-Indication:		FALSE

-	csg-Identity:		not present

Cell B:

-	TAI (MCC/MNC/TAC):		246/080/0002.

-	Access control:		unrestricted.

-	csg-Indication:		TRUE

-	csg-Identity:		04

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and set the UE to automatic PLMN selection mode.

##### 10.1.8.4.2	Procedure

| Step | Direction | Action | Comment | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Run initial activation |  |  |  |
| 2 | UE > USIM | READ EFACSGL, EFOCSGL |  | CR 1 | A.2/1 or A.2/2 |
| 3 | UE > TT | Send RRC CONNECTION REQUEST on Cell A (TAI 246/081/0001) | The TT responds with RRC CONNECTION SETUP |  |  |
| 4 | TT > UE | Send RRC CONNECTION SETUP COMPLETE |  |  |  |
| 5 | UE > TT | Send ATTACH REQUEST | The TT initiates authentication, starts integrity by using the security procedure. |  |  |
| 6 | TT > UE | Send ATTACH ACCEPT with: - TAI (MCC/MNC/TAC): 246/081/ 0001 - GUTI: "24608100010266345678" | UE sends ATTACH COMPLETE, and further TT sends RRC CONNECTION RELEASE |  |  |
| 7 | TT > UE | Use the MMI of the UE to perform manual CSG selection. | The UE shall not indicate the availability of a cell with csg-Identity 04 for PLMN 246/080, this shall be verified for 2 minutes. | CR 3 |  |
| 8 | TT | The network stops all RF output for Cell A (TAI 246/081/0001) | The BCCH of Cell A is reconfigured to indicate the following: - TAI (MCC/MNC/TAC): 246/081/0002 - csg-Indication: TRUE - csg-Identity: 04 TT resumes RF output on the BCCH. |  |  |
| 9 | TT > UE | Use the MMI of the UE to perform manual CSG selection. | The UE shall indicate the availability of a cell with csg-Identity 04 for PLMN 246/081 (Cell A). The user shall select this cell by using the MMI. | CR 3 |  |
| 10 | UE > TT | Send RRC CONNECTION REQUEST on csg cell (Cell A) (TAI 246/081/0002) | The TT responds with RRC CONNECTION SETUP |  |  |
| 11 | TT > UE | Send RRC CONNECTION SETUP COMPLETE |  |  |  |
| 12 | UE > TT | Send TRACKING AREA UPDATE REQUEST | The TT initiates authentication, starts integrity by using the security procedure. |  |  |
| 13 | TT > UE | Send TRACKING AREA UPDATE ACCEPT with the following parameters with integrity protection: - TAI (MCC/MNC/TAC): 246/081/ 0002 - GUTI: "24608100010266345699" | UE sends TRACKING AREA UPDATE COMPLETE, and further TT sends RRC CONNECTION RELEASE. |  |  |
| 14 | UE > USIM | UPDATE EFACSGL | In case the verification of the contents of EFACSGL is not done during the UPDATE command step 14) needs to be executed. | CR 2 | A.2/1 OR A.2/2 |
| 15 | UE > USIM | UE is soft powered down. |  |  |  |
| 16 | USER > UE | READ EFEPSLOCI, EFACSGL. EFOCSGL |  |  | A.2/3 |

#### 10.1.8.5	Acceptance criteria

CR 1 shall be explicitly verified in step 2) by ensuring that UE has successfully read EFACSGL and EFOCSGL,

CR 3 shall be verified:

- At step 7) by ensuring that MMI of the UE shall not indicate to the user CSG cell (CSG ID: 04) for PLMN 246/080 and also

- At step 9) by ensuring that MMI of the UE shall indicate to the user CSG cell (CSG ID: 04) for PLMN 246/081

CR 2 shall be explicitly verified either in step 14) by analysing the UPDATE command used to update EFACSGL (A.2/1 or A.2/2) or at step 16) by reading the EFEPSLOCI, EFACSGL, EFOCSGL and verifying that is contains the following.

EFEPSLOCI (EPS Information)

Logically:

GUTI:							24608100010266345699

Last visited registered TAI:	246/081/0002

EPS update status:			updated

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 0B | F6 | 42 | 16 | 80 | 00 | 01 | 02 | 66 | 43 | 65 | 99 |
|  | B13 | B14 | B15 | B16 | B17 | B18 |
|  | 42 | 16 | 80 | 00 | 02 | 00 |

EFACSGL (Allowed CSG Lists)

Logically:

1st CSG list

PLMN:		246 081 (MCC MNC)

1st CSG list	1st CSG Type indication			02

1st CSG list	1st CSG HNB Name indication	02

1st CSG list	1st CSG CSG ID:					02 (27bit)

1st CSG list	2nd CSG Type indication			03

1st CSG list	2nd CSG HNB Name indication	03

1st CSG list	2nd CSG CSG ID:				03 (27bit)

1st CSG list	3rd CSG Type indication	'xx'		(not checked)

1st CSG list	3rd CSG HNB Name indication	'xx'	(not checked)

1st CSG list	3rd CSG CSG ID:					04 (27bit)

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | A0 | 1D | 80 | 03 | 42 | 16 | 80 | 81 | 06 | 02 | 02 | 00 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 00 | 00 | 5F | 81 | 06 | 03 | 03 | 00 | 00 | 00 | 7F | 81 |
|  | B25 | B26 | B27 | B28 | B29 | B30 | B31 |
|  | 06 | xx | xx | 00 | 00 | 00 | 9F |

2nd CSG list

PLMN:		244 081 (MCC MNC)

2nd CSG list	1st CSG Type indication			08

2nd CSG list	1st CSG HNB Name indication	08

2nd CSG list	1st CSG CSG ID:					08 (27bit)

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | A0 | 0D | 80 | 03 | 42 | 14 | 80 | 81 | 06 | 08 | 08 | 00 |
|  | B13 | B14 | B15 |
|  | 00 | 01 | 1F |

NOTE:	The 1st and 2nd CSG list may be stored together or separately in any record in arbitrary order.

EFOCSGL (Operator CSG Lists)

Unchanged values as defined in 4.5.6.

## 10.2	CSG list handling for UTRA

### 10.2.1	Manual CSG selection without display restrictions in UTRA with ACSG list and OCSG list on USIM

UTRAN test - not applicable

### 10.2.2	Manual CSG selection with display restrictions in UTRA with ACSG list and OCSG list on USIM

UTRAN test - not applicable

### 10.2.3	Manual CSG selection in UTRA with CSG list on USIM, success

UTRAN test - not applicable

# 11	NAS security context parameter handling

## 11.1	NAS security context parameter handling when service "EMM Information" is available

### 11.1.1	Definition and applicability

The security parameters for authentication, integrity protection and ciphering are tied together in an EPS security context and identified by a key set identifier for E-UTRAN (eKSI). The relationship between the security parameters is defined in TS 33.401 [50].

The EPS security context parameters shall be stored on the USIM if the corresponding file is present. If the corresponding file is not present on the USIM, these EMM parameters except allowed CSG list are stored in a non-volatile memory in the ME together with the IMSI from the USIM.

The EFEPSNSC contains the EPS NAS Security context as defined in TS 33.401 [50]. This file shall contain only one record and shall be updated only when the requirements defined in TS 33.401 [50] are met.

### 11.1.2	Conformance requirement

CR 1	The EPS security context parameters shall be stored on the USIM and mark the native EPS security context on the USIM as valid if the corresponding file is present, and shall be updated only when the requirements defined in TS 33.401 [50] are met.

CR 2	The EPS NAS security context parameters shall be stored when the UE transitions to EMM-DEREGISTERED state from EMM-REGISTERED state, as defined in TS 33.401 [50].

Reference:

-	TS 24.301 [21], clause 4.4.2.1 and Annex C;

-	TS 31.102 [19], clause 4.2.92;

-	TS 33.401 [50], clause 6.1.1, 7.2.5.1 and 7.2.5.2.1.

### 11.1.3	Test purpose

The purpose of this test is to verify that the ME generates the EPS security context identified by a key set identifier for E-UTRAN (eKSI) and stores all inside EFEPSNSC if this EF is available and when the requirements defined in TS 33.401 [50], clauses 7.2.5.1 and 7.2.5.2.1 are met.

### 11.1.4	Method of test

#### 11.1.4.1	Initial conditions

The values of the E-UTRAN/EPC-UICC as defined in clause 4.5.4 of the present document are used with EFIMSI as defined in clause 4.6.2.

The TT (E-USS OR NB-SS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):	246/081/0001.

-	Access control:			unrestricted.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case.

#### 11.1.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | USER | Run initial activation. |  |  |  |
| 2 | UE > USIM | Read EFUST, Read record 1 EFEPSNSC |  | CR 1 | A.2/1 OR A.2/2 |
| 3 | UE > TT | RRC CONNECTION REQUEST / RRC CONNECTION REQUEST-NB | TT responds with a RRC CONNECTION SETUP / RRC CONNECTION SETUP-NB |  |  |
| 4 | UE > TT | RRC CONNECTION SETUP COMPLETE / RRC CONNECTION SETUP COMPLETE-NB |  |  |  |
| 5 | UE > TT | ATTACH REQUEST | eKSI: '07' | CR 1 |  |
| 6 | TT > UE | AUTHENTICATION REQUEST | For EPS authentication and AKA procedure TT uses eKSI: '00' |  |  |
| 7 | UE > TT | AUTHENTICATION RESPONSE |  |  |  |
| 8 | UE > TT | SecurityModeCommand | UE responds with a SECURITY MODE COMPLETE |  |  |
| 9 | TT > UE | ATTACH ACCEPT - TAI (MCC/MNC/TAC): 246/081/ 0001 - GUTI "24608100010266345678" |  |  |  |
| 10 | UE > TT | ATTACH COMPLETE |  |  |  |
| 11 | TT > UE | Send a RRC CONNECTION RELEASE / RRC CONNECTION RELEASE-NB |  |  |  |
| 12 | UE > USIM | READ RECORD 1 of EFEPSNSC |  | CR 2 | A.2/1 OR A.2/2 |
| 13 | UE > TT | The UE is soft powered down, and initiates DETACH procedure. |  |  |  |
| 14 | UE > USIM | UPDATE EFEPSNSC |  | CR 1CR 2 | A.2/1 OR A.2/2 OR A.2/3 |

#### 11.1.4.5	Acceptance criteria

CR1 is explicitly verified at step 2) before UE attempts registration by ensuring that UE shall read EFUST and EFEPSNSC, and also at Step 14) after detach is performed by ensuring that UE shall update the EFEPSNSC.

At step 2), EFEPSNSC shall contain:

EFEPSNSC (EPS NAS Security Context)

Logically:

Key Set Identifier KSIASME:		'07'

ASME Key (KSIASME):			32 byte key, value not checked

Uplink NAS count:				any value

Downlink NAS count:			any value

Identifiers of selected NAS		any value

integrity and encryption algorithm

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | A0 | xx | 80 | 01 | 07 | 81 | xx | xx |
|  | … | Bxx |
|  | … | xx |

At step 14), EFEPSNSC shall contain:

EFEPSNSC (EPS NAS Security Context)

Logically:

Key Set Identifier KSIASME:		'00'

ASME Key (KSIASME):			32 byte key, value not checked

Uplink NAS count:				any value

Downlink NAS count:			any value

Identifiers of selected NAS		any value

integrity and encryption algorithm

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | A0 | xx | 80 | 01 | 00 | 81 | xx | xx |
|  | … | Bxx |
|  | … | xx |

CR 1 is also verified at step 5) by analysing the ATTACH REQUEST and UE indicated that no key is available.

CR 2 is explicitly verified at step 12) and ensure that EFEPSNSC remains unchanged from initial value.

## 11.2	NAS security context parameter handling when service "EMM Information" is not available, no IMSI change

### 11.2.1	Definition and applicability

The security parameters for authentication, integrity protection and ciphering are tied together in an EPS security context and identified by a key set identifier for E-UTRAN (eKSI). The relationship between the security parameters is defined in TS 33.401 [50].

The EPS security context parameters shall be stored on the USIM if the corresponding file is present. If the corresponding file is not present on the USIM, these EMM parameters except allowed CSG list are stored in a non-volatile memory in the ME together with the IMSI from the USIM.

The EFEPSNSC contains the EPS NAS Security context as defined in TS 33.401 [50]. This file shall contain only one record.

### 11.2.2	Conformance requirement

CR 1	The EPS security context parameters shall be stored on the non-volatile memory in the ME together with the IMSI from the USIM if the corresponding file to store EPS security context parameters is not present on the USIM.

Reference:

-	TS 24.301 [21], clause 4.4.2.1 and Annex C;

-	TS 31.102 [19], clause 4.2.92;

-	TS 33.401 [50], clause 6.1.1.

CR 2	These EMM parameters can only be used if the IMSI from the USIM matches the IMSI stored in the non-volatile memory; else the UE shall delete the EMM parameters.

Reference:

-	TS 24.301 [21], clause 4.4.2.1 and Annex C;

-	TS 33.401 [50], clause 6.1.1.

### 11.2.3	Test purpose

The purpose of this test is to verify that the ME generates the EPS security context identified by a key set identifier for E-UTRAN (eKSI) and stores all inside a non-volatile memory in the ME as EMM information is not available on the USIM. During the test the IMSI on the USIM remains unchanged.

### 11.2.4	Method of test

#### 11.2.4.1	Initial conditions

The values of the Default UICC as defined in clause 4.5.2 of the present document are used with the following exception:

EFUST (USIM Service Table)

Logically:

| Service n°85: |  | EPS Mobility Management Information | not available |
| --- | --- | --- | --- |

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary | xx1x xx11 | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxxx xxxx | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | B12 |
|  | xxxx xxxx | xxxx xxxx | xx10 xxxx | 0000 0xxx |

The TT (E-USS OR NB-SS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):	246/081/0001.

-	Access control:			unrestricted.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case.

#### 11.2.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | USER | Run initial activation. |  |  |  |
| 2 | UE > USIM | READ EFUST |  | CR 1 | A.2/1 OR A.2/2 |
| 3 | UE > TT | RRC CONNECTION REQUEST / RRC CONNECTION REQUEST-NB | TT responds with a RRC CONNECTION SETUP / RRC CONNECTION SETUP-NB |  |  |
| 4 | UE > TT | RRC CONNECTION SETUP COMPLETE / RRC CONNECTION SETUP COMPLETE-NB |  |  |  |
| 5 | UE > TT | ATTACH REQUEST | eKSI: '07' |  |  |
| 6 | TT > UE | AUTHENTICATION REQUEST | For EPS authentication and AKA procedure TT uses eKSI: '00' |  |  |
| 7 | UE > TT | AUTHENTICATION RESPONSE |  |  |  |
| 8 | UE > TT | SecurityModeCommand | TT responds with a SECURITY MODE COMPLETE |  |  |
| 9 | TT > UE | ATTACH ACCEPT - TAI (MCC/MNC/TAC): 246/081/ 0001 - GUTI: "24608100010266345678" |  |  |  |
| 10 | UE > TT | ATTACH COMPLETE |  |  |  |
| 11 | TT > UE | RRC CONNECTION RELEASE / RRC CONNECTION RELEASE-NB |  |  |  |
| 12 | UE > TT | The UE is powered off, and initiates Detach procedure. |  |  |  |
| 13 | USER | Run initial activation |  |  |  |
| 14 | UE > TT | RRC CONNECTION REQUEST / RRC CONNECTION REQUEST-NB | TT responds with a RRC CONNECTION SETUP / RRC CONNECTION SETUP-NB |  |  |
| 15 | UE > TT | RRC CONNECTION SETUP COMPLETE / RRC CONNECTION SETUP COMPLETE-NB |  |  |  |
| 16 | UE > TT | ATTACH REQUEST | eKSI: '00' | CR 2 |  |
| 17 | TT > UE | SecurityModeCommand (TT uses the last known KASME) | UE responds with a SECURITY MODE COMPLETE | CR 2 |  |
| 18 | TT > UE | ATTACH ACCEPT - TAI (MCC/MNC/TAC): 246/081/0001 - GUTI: "24608100010266345619" |  |  |  |
| 19 | UE > TT | ATTACH COMPLETE |  |  |  |
| 20 | TT > UE | RRC CONNECTION RELEASE / RRC CONNECTION RELEASE-NB |  |  |  |
| 21 | USER | The UE is powered down |  |  |  |

### 11.2.5	Acceptance criteria

CR1 is explicitly verified at step 2) by verifying that UE read EFUST

CR 2 can be verified at step 16) by ensuring that the UE has indicated in ATTACH REQUEST eKSI as '00' and NAS security mode procedure is successful at step 17).

## 11.3	NAS security context parameter handling when service "EMM Information" is not available, IMSI Changed

### 11.3.1	Definition and applicability

The security parameters for authentication, integrity protection and ciphering are tied together in an EPS security context and identified by a key set identifier for E-UTRAN (eKSI). The relationship between the security parameters is defined in TS 33.401 [50].

The EPS security context parameters shall be stored on the USIM if the corresponding file is present. If the corresponding file is not present on the USIM, these EMM parameters except allowed CSG list are stored in a non-volatile memory in the ME together with the IMSI from the USIM.

The EFEPSNSC contains the EPS NAS Security context as defined in TS 33.401 [50]. This file shall contain only one record.

### 11.3.2	Conformance requirement

CR 1	The EPS security context parameters shall be stored on the non-volatile memory in the ME together with the IMSI from the USIM if the corresponding file to store EPS security context parameters is not present on the USIM.

Reference:

-	TS 24.301 [21], clause 4.4.2.1 and Annex C;

-	TS 31.102 [19], clause 4.2.92;

-	TS 33.401 [50], clause 6.1.1.

CR 2	These EMM parameters can only be used if the IMSI from the USIM matches the IMSI stored in the non-volatile memory; else the UE shall delete the EMM parameters.

Reference:

-	TS 24.301 [21], clause 4.4.2.1 and Annex C;

-	TS 33.401 [50], clause 6.1.1.

### 11.3.3	Test purpose

The purpose of this test is to verify that:

1)	the ME generates the EPS security context identified by a key set identifier for E-UTRAN (eKSI) and stores all inside a non-volatile memory in the ME as EMM information is not available on the USIM.

2)	the UE deletes existing EMM parameters from the ME's non-volatile memory in case a different IMSI is activated.

### 11.3.4	Method of test

#### 11.3.4.1	Initial conditions

The values of the Default UICC as defined in clause 4.5.2 of the present document are used with the following exception:

EFUST (USIM Service Table)

Logically:

| Service n°85: |  | EPS Mobility Management Information | not available |
| --- | --- | --- | --- |

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary | xx1x xx11 | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxxx xxxx | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | B12 |
|  | xxxx xxxx | xxxx xxxx | xx10 xxxx | 0000 0xxx |

The TT (E-USS OR NB-SS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):	246/081/0001.

-	Access control:			unrestricted.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case.

#### 11.3.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | USER | - Run initial activation. |  |  |  |
| 2 | UE > USIM | - READ EFUST |  | CR 1 | A.2/1 OR A.2/2 |
| 3 | UE > TT | - RRC CONNECTION REQUEST / RRC CONNECTION REQUEST-NB | TT responds with a RRC CONNECTION SETUP / RRC CONNECTION SETUP-NB |  |  |
| 4 | UE > TT | - RRC CONNECTION SETUP COMPLETE / RRC CONNECTION SETUP COMPLETE-NB |  |  |  |
| 5 | UE > TT | - ATTACH REQUEST | eKSI: '07' |  |  |
| 6 | TT > UE | - AUTHENTICATION REQUEST | For EPS authentication and AKA procedure TT uses eKSI: '00' |  |  |
| 7 | UE > TT | - AUTHENTICATION RESPONSE |  |  |  |
| 8 | UE > TT | - SecurityModeCommand | TT responds with a SECURITY MODE COMPLETE |  |  |
| 9 | TT > UE | ATTACH ACCEPT - TAI (MCC/MNC/TAC): 246/081/ 0001 - GUTI: "24608100010266345678" |  |  |  |
| 10 | UE > TT | ATTACH COMPLETE |  |  |  |
| 11 | TT > UE | - RRC CONNECTION RELEASE / RRC CONNECTION RELEASE-NB |  |  |  |
| 12 | UE > TT | - The UE is powered off, and initiates DETACH procedure. |  |  |  |
| 13 | UE | - Run initial activation with IMSI set to "246081222233333". |  |  |  |
| 14 | UE > TT | - RRC CONNECTION REQUEST / RRC CONNECTION REQUEST-NB | TT responds with a RRC CONNECTION SETUP / RRC CONNECTION SETUP-NB |  |  |
| 15 | UE > TT | - RRC CONNECTION SETUP COMPLETE / RRC CONNECTION SETUP COMPLETE-NB |  |  |  |
| 16 | UE > TT | - ATTACH REQUEST | eKSI: '07' | CR 2 |  |
| 17 | TT > UE | - SecurityModeCommand (TT uses the last known KASME) | UE responds with a SECURITY MODE REJECT | CR 2 |  |
| 20 | TT > UE | RRC CONNECTION RELEASE / RRC CONNECTION RELEASE-NB |  |  |  |
| 21 | USER | The UE is powered down |  |  |  |

### 11.3.5	Acceptance criteria

CR1 is explicitly verified at step 2) by verifying that UE read EFUST

CR 2 can be verified at step 16) by ensuring that the UE has indicated in ATTACH REQUEST eKSI as '07' and NAS security mode procedure is rejected by ME at step 17).

## 11.4	EPS NAS Security Context Storage

### 11.4.1	Definition and applicability

The security parameters for authentication, integrity protection and ciphering are tied together in an EPS security context and identified by a key set identifier for E-UTRAN (eKSI). The relationship between the security parameters is defined in 3GPP TS 33.401 [27].

The EPS security context parameters shall be stored on the USIM if the corresponding file is present. If the corresponding file is not present on the USIM, these EMM parameters except allowed CSG list are stored in a non-volatile memory in the ME together with the IMSI from the USIM.

The EFEPSNSC contains the EPS NAS Security context as defined in TS 33.401 [27]. This file shall contain only one record and shall be updated only when the requirements defined in TS 33.401 [27] are met.

### 11.4.2	Conformance requirement

CR 1	The EPS security context parameters shall be stored on the USIM and mark the native EPS security context on the USIM as valid if the corresponding file is present, and shall be updated only when the requirements defined in TS 33.401 [27] are met.

Reference:

-	TS 24.301 [21], clause 4.4.2.1 and Annex C;

-	TS 31.102 [19], clause 4.2.92 and 5.2.28;

-	TS 33.401 [50], clause 6.1.1, 7.2.5.2, 7.2.6.1 and 7.2.6.3.

CR 2	The initial NAS message shall be integrity protected by the current EPS NAS security context if such exists. If no current EPS NAS security context exists the ME shall signal “no key available” in the initial NAS message, as defined in TS 33.401 [27].

Reference:

-	TS 24.301 [21], clause 4.4.2.1 and Annex C;

-	TS 31.102 [19], clause 4.2.92 and 5.2.28;

-	TS 33.401 [50], clause 6.1.1, 7.2.5.2, 7.2.6.1 and 7.2.6.3.

CR 3	ME shall handle the EPS NAS security context in accordance with the rules and procedures as specified in TS 33.401 [27] during ECM state transition.

Reference:

-	TS 33.401 [50], clause 6.1.1, 7.2.5.2, 7.2.6.1 and 7.2.6.3.

### 11.4.3	Test purpose

The purpose of this test is to verify that the update of EPS NAS security context shall be according to the rules and procedures specified in TS 33.401 [50], clauses 6.1.1, 7.2.5.2, 7.2.6.1 and 7.2.6.3.

### 11.4.4	Method of test

#### 11.4.4.1	Initial conditions

The values of the Default UICC as defined in clause 4.5.4 of the present document are used

The TT (E-USS OR NB-SS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):	246/081/0001.

-	Access control:			unrestricted.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case.

#### 11.4.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | USER | - Run initial activation. |  |  |  |
| 2 | UE > USIM | - READ EFUST and EFEPSNSC |  | CR 1 | A.2/1 OR A.2/2 |
| 3 | UE > TT | RRC CONNECTION REQUEST / RRC CONNECTION REQUEST-NB | TT responds with a RRC CONNECTION SETUP / RRC CONNECTION SETUP-NB |  |  |
| 4 | UE > TT | RRC CONNECTION SETUP COMPLETE / RRC CONNECTION SETUP COMPLETE-NB |  |  |  |
| 5 | UE > TT | ATTACH REQUEST | eKSI: '07' | CR 2 |  |
| 6 | TT > UE | AUTHENTICATION REQUEST | For EPS authentication and AKA procedure TT uses eKSI: '00' | CR 3 | A.2/1 OR A.2/2 |
| 7 | UE > TT | AUTHENTICATION RESPONSE |  | CR 3 | A.2/1 OR A.2/2 |
| 8 | UE > TT | SecurityModeCommand | TT responds with a SECURITY MODE COPLETE | CR 3 | A.2/1 OR A.2/2 |
| 9 | TT > UE | ATTACH ACCEPT with: - TAI (MCC/MNC/TAC): 246/081/ 0001 - GUTI: "24608100010266345678" |  | CR 3 | A.2/1 OR A.2/2 |
| 10 | UE > TT | ATTACH COMPLETE |  | CR 3 | A.2/1 OR A.2/2 |
| 11 | TT > UE | RRC CONNECTION RELEASE / RRC CONNECTION RELEASE-NB |  | CR 3 | A.2/1 OR A.2/2 |
| 12 | UE > TT | PAGING / PAGING-NB using the S-TMSI | CN domain indicator set to ''PS''. | CR 3 | A.2/1 OR A.2/2 |
| 14 | UE > TT | RRC CONNECTION REQUEST / RRC CONNECTION REQUEST-NB | TT responds with a RRC CONNECTION SETUP / RRC CONNECTION SETUP-NB | CR 3 | A.2/1 OR A.2/2 |
| 15 | UE > TT | RRC CONNECTION SETUP COMPLETE / RRC CONNECTION SETUP COMPLETE-NB |  | CR 3 | (A.2/1 OR A.2/2) |
| 16 | UE > TT | EMM SERVICE REQUEST (for WB-S1) CONTROL PLANE SERVICE REQUEST (for NB-IOT) | For WB-S1: TT Activates AS security by the E-USS and the Dedicated EPS bearer is established. For NB-IOT: TT responds with SERVICE ACCEPT and default bearer is established. | CR 3 | A.2/1 OR A.2/2 |
| 17 |  | Wait 5 seconds |  |  |  |
| 18 | TT > UE | RRC CONNECTION RELEASE / RRC CONNECTION RELEASE-NB |  | CR 3 | A.2/1 OR A.2/2 |

### 11.4.5	Acceptance criteria

CR 1 is explicitly verified at step 2) by verifying that UE reads EFUST and EFEPSNSC

CR 2 can be verified at step 5) by ensuring that the UE has indicated in ATTACH REQUEST eKSI as '07' and at step 18) by ensuring NAS security mode procedure is rejected by ME.

CR 3 is explicitly verified at steps 6) to 16) by verifying that UE shall not update EFEPSNSC. This can be alternatively verified by explicitly reading and analysing the value of EFEPSNSC at step 15).

# 12	Non Access Stratum (NAS) Configuration parameter handling

## 12.1	EFNASCONFIG - NAS signalling priority handling

The test case is covered in TS 34.123-1 [43] clauses 9.4.3.7, 9.4.5.5, 11.1.1.3, 11.1.1.4, 11.1.3.4, 11.2.2.3, 12.4.1.1e and 12.4.3.2a. See CP-170166, CR #0229 to TS 31.121 [2] for details

## 12.2	EFNASCONFIG - NMO I Network Mode of Operation I handling

The test case is covered in TS 34.123-1 [43]clause 12.2.2.3a. See CP-170166, CR #0230 to TS 31.121 [2] for details.

## 12.3	EFNASCONFIG - Attach with IMSI handling

The test case is covered in TS 34.123-1 [43] clause 12.2.1.1a. See CP-170166, CR #0224 to TS 31.121 [2] for details.

## 12.4	EFNASCONFIG - Verifying Minimum Periodic Search Timer

The test case is covered in TS 34.123-1 [43] clauses 9.4.5.4 and 9.4.5.4.7. See CP-170166, CR #0228 to TS 31.121 [2] for details.

## 12.5	EFNASCONFIG - Extended access barring handling

The test case is covered in TS 34.123-1 [43] clauses 8.1.1.20, 9.4.11, 12.2.1.16 and 12.4.1.9. See CP-170166, CR #0226 to TS 31.121 [2] for details.

## 12.6	EFNASCONFIG - Verifying Timer T3245 Behaviour

UTRAN test - not applicable

## 12.7	EFNASCONFIG - Override NAS signalling low priority

UTRAN test - not applicable

## 12.8	EFNASCONFIG - Override Extended access barring

UTRAN test - not applicable

## 12.9	EFNASCONFIG - Fast First Higher Priority PLMN Search

UTRAN test - not applicable

## 12.10	EFNASCONFIG - E-UTRA Disabling Allowed for EMM cause #15

Test using UTRAN – not applicable

## 12.11	EFNASCONFIG - SM_RetryWaitTime

UTRAN test - not applicable

## 12.12	EFNASCONFIG - SM_RetryAtRATChange

FFS

# 13	UICC interface during PSM

## 13.1	UICC interface in PSM handling for E-UTRAN – No UICC deactivation in PSM

### 13.1.1	Definition and applicability

PSM is intended for UEs that are expecting only infrequent mobile originating and terminating services and that can accept a corresponding latency in the mobile terminating communication. In order to reduce power consumption while in PSM, and only in case the PIN of the USIM is disabled, the ME may optionally deactivate the UICC after entering the PSM.

### 13.1.2	Conformance requirement

CR 1	In order to reduce power consumption while the ME is in PSM, and only in case the PIN of the USIM is disabled, the ME may optionally deactivate the UICC (as specified in clause 6A.1 of TS 31.101 [33]) after entering the PSM.

Reference:

-	TS 31.102 [19], clause 5.1.10;

-	TS 24.301 [21], clauses 5.3.5 and 5.3.11.

-	TS 31.101 [33], clause 6A.1.

### 13.1.3	Test purpose

To verify that UE does not deactivate the UICC in case the PIN for the USIM is enabled and verified.

### 13.1.4	Method of test

#### 13.1.4.1	Initial conditions

The values of the E-UTRAN/EPC UICC as defined in clause 4.5.4 of the present document are used with the following exceptions:

The PIN of the USIM is enabled.

EFUMPC

Logically:

UICC maximum power consumption:	60 mA

Operator defined time out (T_OP):		5 seconds

Additional information:					UICC does not require increased idle current
												UICC does not support the UICC suspension procedure

Byte 4 and byte 5:						RFU

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 |
| --- | --- | --- | --- | --- | --- |
| Hex | 3C | 05 | 00 | 00 | 00 |

The UE is configured to use Power Saving Mode.

The UE is configured to use the timer T3324 set to T3324_V.

The TT (E-USS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):	246/081/0001.

-	Access control:	unrestricted.

The TT (NB-SS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):	246/081/0001.

-	Access control:	unrestricted.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case.

#### 13.1.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Run initial activation. |  |  |  |
| 2 | USER > UE | Enter PIN when in PIN check mode | The PIN 0000 is used |  |  |
| 3 | UE > TT | Send RRC CONNECTION REQUEST / RRC CONNECTION REQUEST-NB | TT responds with RRC CONNECTION SETUP / RRC CONNECTION SETUP-NB |  |  |
| 3 | UE > TT | The UE transmits an ATTACH REQUEST message including T3324 set to T3324_V | The TT sends the ATTACH ACCEPT message contains T3324 set to T3324_V and T3412 set to T3412_V. The ATTACH ACCEPT message shall not contain the eDRX parameters |  |  |
| 4 | UE > TT | The UE transmits the ATTACH COMPLETE during registration from the UE | The TT sends RRC CONNECTION RELEASE / RRC CONNECTION RELEASE-NB. | CR 1 |  |
| 5 | UE | The UE is waiting for the T3412 timer expiration |  | CR 1 |  |
| 6 | UE | The UE sends TRACKING AREA UPDATE REQUEST | The TT sends TRACKING AREA UPDATE ACCEPT | CR 1 |  |
| 7 | UE | The UE is powered off. |  |  |  |

### 13.1.5	Acceptance criteria

CR 1 is met if the UE does not deactivate the UICC or send SUSPEND UICC command after step 4).

## 13.2	UICC interface in PSM handling for E-UTRAN – PSM not accepted by E-USS/NB-SS

### 13.2.1	Definition and applicability

PSM is intended for UEs that are expecting only infrequent mobile originating and terminating services and that can accept a corresponding latency in the mobile terminating communication. In order to reduce power consumption while in PSM, and only in case the PIN of the USIM is disabled, the ME may optionally deactivate the UICC after entering the PSM.

### 13.2.2	Conformance requirement

CR 1	In order to reduce power consumption while the ME is in PSM, and only in case the PIN of the USIM is disabled, the ME may optionally deactivate the UICC (as specified in clause 6A.1 of TS 31.101 [33]) after entering the PSM.

Reference:

-	TS 31.102 [19], clause 5.1.10;

-	TS 24.301 [21], clauses 5.3.5 and 5.3.11.

-	TS 31.101 [33] in clause 6A.1.

### 13.2.3	Test purpose

The purpose of this test is to verify that the UE does not deactivate the UICC in case the network is not supporting/accepting PSM.

### 13.2.4	Method of test

#### 13.2.4.1	Initial conditions

The UE is configured to use Power Saving Mode.

The UE is configured to use the timer T3324 set to T3324_V.

The E-USS transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):	246/081/0001.

-	Access control:	unrestricted.

The NB-SS transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):	246/081/0001.

-	Access control:	unrestricted.

The default E-UTRAN UICC is installed into the Terminal.

The PIN of the USIM is disabled.

#### 13.2.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Run initial activation. |  |  |  |
| 2 | UE > TT | The UE requests RRC Connection | The TT responds with RCC Connection setup |  |  |
| 3 | UE > TT | The UE transmits an ATTACH REQUEST message including T3324 set to T3324_V. |  |  |  |
| 4 | TT > UE | The TT sends the ATTACH ACCEPT message contains T3324 set to "deactivated". | The ATTACH ACCEPT message shall not contain the eDRX parameters | CR1 |  |
| 5 | UE > TT | The UE transmits the ATTACH COMPLETE during registration from the UE | The TT sends RRC CONNECTION RELEASE/RRC CONNECTION RELEASE-NB. | CR1 |  |
| 6 | TT | The TT is waiting for the T3324_V time period to pass |  | CR1 |  |
| 7 | TT > UE | The TT transmits PAGING / PAGING-NB using the S-TMSI. |  | CR1 |  |
| 8 | UE > TT | Send RRC CONNECTION REQUEST / RRC CONNECTION REQUEST-NB | The TT responds with a RRC CONNECTION SETUP / RRC CONNECTION SETUP-NB | CR1 |  |
| 9 | UE > TT | Send RRC CONNECTION SETUP COMPLETE / RRC CONNECTION SETUP COMPLETE-NB |  | CR1 |  |
| 10 | UE > TT | The UE sends SERVICE REQUEST | The TT sends SERVICE ACCEPT followed by RRC CONNECTION RELEASE/RRC CONNECTION RELEASE-NB | CR1 |  |
| 11 | UE | - The UE is powered off. |  |  |  |

### 13.2.5	Acceptance criteria

CR 1 is met if the UE does not deactivate the UICC or send SUSPEND UICC command after step 4).

NOTE:	The execution of the SUSPEND UICC command is no subject to be tested in this sequence.

## 13.3	UICC interface in PSM handling for E-UTRAN – UICC deactivation in PSM

A reliable verification of the activation/deactivation of the UICC in PSM can be done with a physical connection to supply contacts only. Unless a suitable and sufficient environment is introduced the definition of this test is FFS.

## 13.4	UICC interface in PSM for E-UTRAN – SUSPEND UICC

### 13.4.1	Definition and applicability

PSM is intended for UEs that are expecting only infrequent mobile originating and terminating services and that can accept a corresponding latency in the mobile terminating communication. If the UICC supports the UICC suspension mechanism (SUSPEND UICC command), the ME may suspend the UICC after entering the PSM. In this case, the ME shall successfully resume the UICC before it can leave the PSM.

### 13.4.2	Conformance requirement

CR 1	If the UICC supports the UICC suspension mechanism (SUSPEND UICC command), the ME may suspend the UICC after entering the PSM. In this case, the ME shall successfully resume the UICC before it can leave the PSM.

Reference:

-	TS 31.102 [19], clause 5.1.10;

-	TS 24.301 [21], clauses 5.3.5, 5.3.11 and 5.5.1.2.2.

-	TS 31.101 [33], clause 11.1.22.

### 13.4.3	Test purpose

1)	To verify that after reading EFUMPC and entering the PSM the ME sends a SUSPEND UICC command to the UICC in case the UICC indicates the support of the SUSPEND UICC command.

2)	To verify that the ME resumes the UICC before it can leave the PSM.

### 13.4.4	Method of test

#### 13.4.4.1	Initial conditions

The UE is configured to use Power Saving Mode.

The UE is configured to use the timer T3324 set to T3324_V.

The E-USS transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):	246/081/0001.

-	Access control:	unrestricted.

The NB-SS transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):	246/081/0001.

-	Access control:	unrestricted.

The default E-UTRAN UICC is used with the following exceptions:

EFUMPC	(UICC Maximum Power Consumption)

Logically:

UICC maximum power consumption:	60 mA

Operator defined time out (T_OP):	5 seconds

Additional information:	UICC does not require increased idle current

UICC supports the UICC suspension procedure

Byte 4 and byte 5:	RFU

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 |
| --- | --- | --- | --- | --- | --- |
| Hex | 3C | 05 | 00 | 00 | 00 |

The PIN of the USIM is enabled and verified.

#### 13.4.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Run initial activation. |  |  |  |
| 2 | UE > TT | The UE requests RRC Connection | The TT responds with RCC Connection setup |  |  |
| 3 | UE > TT | The UE transmits an ATTACH REQUEST message including T3324 set to T3324_V. |  |  |  |
| 4 | TT > UE | The TT sends the ATTACH ACCEPT message contains T3324 set to T3324_V and T3412 set to T3412_V. | The ATTACH ACCEPT message shall not contain the eDRX parameters | CR1 |  |
| 5 | UE > TT | The UE transmits the ATTACH COMPLETE during registration from the UE | The TT sends RRC CONNECTION RELEASE/RRC CONNECTION RELEASE-NB. | CR1 |  |
| 6 | ME > UICC | The ME sends SUSPEND UICC command to the UICC indicating "Minimum duration of the suspension proposed by the terminal" and the "Maximum duration of the suspension proposed by the terminal" | The UICC returns a maximum suspension duration in the range proposed by the ME, Resume token and SW 9000. |  |  |
| 7 | UE | The UE is waiting for the T3412 timer expiration |  | CR1 |  |
| 8 | UE | The UE sends TRACKING AREA UPDATE REQUEST | The TT sends TRACKING AREA UPDATE ACCEPT | CR1 |  |
| 9 | UE | The UE is powered off. |  |  |  |

NOTE:	For testing purposes, and to not extend the test execution time the maximum suspension duration returned by the UICC shall be equal to the minimum duration of the suspension proposed by the terminal.

### 13.4.5	Acceptance criteria

CR 1 is met if the ME:

- reads EFUMPC after step 1) to verify that the SUSPEND UICC command is supported.

Note: RFU – agreed method to verify contents of EFUMPC

2)	deactivates the UICC after Step (6) as specified in 3GPP TS 31.101 [39].

3)	leaves the PSM and resumes the UICC after Step (8).

# 14	UICC interface during eDRX

## 14.1	UICC interface during eDRX for E-UTRAN – eDRX is not supported by the UICC

### 14.1.1	Definition and applicability

In order to reduce power consumption when the UE uses extended idle mode DRX cycle, the UE may optionally deactivate the UICC  during the extended idle mode DRX cycle.

### 14.1.2	Conformance requirement

CR 1	In case the UICC does not support the UICC suspension mechanism, the PIN of the USIM is disabled and deactivation of UICC is authorized in EFAD, the UE may optionally deactivate the UICC (as specified in clause 6A.1 of TS 31.101 [33]) during the extended idle mode DRX cycle.

Reference:

-	TS 31.102 [19], clause 5.1.11;

-	TS 24.301 [21], clauses 5.3.12.

-	TS 23.401 [42], clause 5.13a.

-	TS 31.101 [33], clause 6A.1.

### 14.1.3	Test purpose

The purpose of this test is to verify that the UE does not deactivate the UICC in case the ME is not authorized to modify the polling interval and/or disable the UICC interface during extended DRX cycle in EFAD in USIM.

### 14.1.4	Method of test

#### 14.1.4.1	Initial conditions

The UE is configured to request the use of eDRX (in the ATTACH REQUEST and TrackingAreaUpdate messages).

The E-USS transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):	246/081/0001.

-	Access control:	unrestricted.

The NB-SS transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):	246/081/0001.

-	Access control:	unrestricted.

The default E-UTRAN UICC is used with the following exceptions:

EFUMPC	(UICC Maximum Power Consumption)

Logically:

UICC maximum power consumption:	60 mA

Operator defined time out (T_OP):	5 seconds

Additional information:	UICC does not require increased idle current

UICC does not support the UICC suspension procedure

Byte 4 and byte 5:	RFU

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 |
| --- | --- | --- | --- | --- | --- |
| Hex | 3C | 05 | 00 | 00 | 00 |

The PIN of the USIM is enabled and verified.

#### 14.1.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Run initial activation. |  |  |  |
| 2 | UE > TT | The UE requests RRC Connection | The TT responds with RCC Connection setup |  |  |
| 3 | UE > TT | The UE transmits an ATTACH REQUEST message including the eDRX parameters |  |  |  |
| 4 | TT > UE | The TT sends the ATTACH ACCEPT message containing the eDRX parameters. | If ATTACH REQUEST in step 3) above also contains T3324, the ATTACH ACCEPT message shall contain T3324 set to "deactivated". If ATTACH REQUEST in step 3) does not contain T3324, the ATTACH ACCEPT message shall not contain T3324. |  |  |
| 5 | UE > TT | The UE transmits the ATTACH COMPLETE during registration from the UE | The TT sends RRC CONNECTION RELEASE / RRC CONNECTION RELEASE-NB. | CR1 |  |
| 6 | TT > UE | The TT transmits PAGING / PAGING-NB using the S-TMSI in a valid paging occasion within the PTW of the paging Hyperframes as per Idle eDRX. |  | CR1 |  |
| 7 | UE > TT | Send RRC CONNECTION REQUEST / RRC CONNECTION REQUEST-NB | The TT responds with a RRC CONNECTION SETUP / RRC CONNECTION SETUP-NB | CR1 |  |
| 8 | UE > TT | Send RRC CONNECTION SETUP COMPLETE / RRC CONNECTION SETUP COMPLETE-NB |  | CR1 |  |
| 9 | UE > TT | The UE sends ServiceRequest | The TT sends ServiceAccept followed by RRC CONNECTION RELEASE / RRC CONNECTION RELEASE-NB | CR1 |  |
| 10 | UE | - The UE is powered off. |  |  |  |

### 14.1.5	Acceptance criteria

CR 1 is met if the UE does not deactivate the UICC or send SUSPEND UICC command after step 5).

## 14.2	UICC interface during eDRX for E-UTRAN – eDRX is not accepted by E-USS/NB-SS

### 14.2.1	Definition and applicability

In order to reduce power consumption when the UE uses extended idle mode DRX cycle, the UE may optionally deactivate the UICC  during the extended idle mode DRX cycle.

### 14.2.2	Conformance requirement

CR 1	In case the UICC does not support the UICC suspension mechanism, the PIN of the USIM is disabled and deactivation of UICC is authorized in EFAD, the UE may optionally deactivate the UICC (as specified in clause 6A.1 of 3GPP TS 31.101 [33]) during the extended idle mode DRX cycle.

Reference:

-	TS 31.102 [19], clause 5.1.11;

-	TS 24.301 [21], clauses 5.3.12;

-	TS 23.401 [42], clause 5.13a;

-	TS 31.101 [33], clause 6A.1.

### 14.2.3	Test purpose

1)	To verify that UE does not deactivate the UICC in case extended DRX cycle is not supported by the network

### 14.2.4	Method of test

#### 14.2.4.1	Initial conditions

The UE is configured to request the use of eDRX (in the ATTACH REQUEST and TrackingAreaUpdate messages).

The E-USS transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):	246/081/0001.

-	Access control:	unrestricted.

The NB-SS transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):	246/081/0001.

-	Access control:	unrestricted.

The default E-UTRAN UICC is used with the following exceptions:

EFAD	(Administrative Data)

Logically:	Normal operation + specific facilities;

Ciphering indicator feature disabled;

CSG Display Control: for every PLMN not included in EF_OCSGL, or for which a CSG display indicator tag is not present, all available CSGs can be displayed without any restriction;

ProSe services for Public Safety: the ME is not authorized for ProSe services for Public Safety usage without contacting the ProSe Function;

Extended DRX cycle: the ME is authorized to modify the polling interval and/or disable the UICC interface during extended DRX cycle;

Length of MNC in the IMSI: 3.

Coding:

| Byte | B1 | B2 | B3 | B4 |
| --- | --- | --- | --- | --- |
| Hex | 01 | 00 | 08 | 03 |

The PIN of the USIM is disabled.

#### 14.2.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Run initial activation. |  |  |  |
| 2 | UE > TT | The UE requests RRC Connection | The TT responds with RCC Connection setup |  |  |
| 3 | UE > TT | The UE transmits an ATTACH REQUEST message including the eDRX parameters |  |  |  |
| 4 | TT > UE | The TT sends the ATTACH ACCEPT message containing the eDRX parameters. | If ATTACH REQUEST in step 3) above also contains T3324, the ATTACH ACCEPT message shall contain T3324 set to "deactivated". If ATTACH REQUEST in step 3) does not contain T3324, the ATTACH ACCEPT message shall not contain T3324. |  |  |
| 5 | UE > TT | The UE transmits the ATTACH COMPLETE during registration from the UE | The TT sends RRC CONNECTION RELEASE/RRC CONNECTION RELEASE-NB. | CR1 |  |
| 6 | TT > UE | The TT transmits PAGING / PAGING-NB in a valid paging occasion as per normal DRX. |  | CR1 |  |
| 7 | UE > TT | Send RRC CONNECTION REQUEST / RRC CONNECTION REQUEST-NB | The TT responds with a RRC CONNECTION SETUP / RRC CONNECTION SETUP-NB | CR1 |  |
| 8 | UE > TT | Send RRC CONNECTION SETUP COMPLETE / RRC CONNECTION SETUP COMPLETE-NB |  | CR1 |  |
| 9 | UE > TT | The UE sends ServiceRequest | The TT sends ServiceAccept followed by RRC CONNECTION RELEASE/RRC CONNECTION RELEASE-NB | CR1 |  |
| 10 | UE | - The UE is powered off. |  |  |  |

### 14.2.5	Acceptance criteria

CR 1 is met if the UE does not deactivate the UICC or send SUSPEND UICC command after step 5).

## 14.3	UICC interface during eDRX for E-UTRAN – UICC deactivation during eDRX

FFS

## 14.4	UICC interface during eDRX for E-UTRAN– SUSPEND UICC

FFS

# 15	Authentication procedure and NAS security context handling for 5G

## 15.1	Authentication procedure for EAP-AKA'

### 15.1.1	Authentication procedure for EAP-AKA' - Authentication is successful

#### 15.1.1.1	Definition and applicability

The purpose of the primary authentication and key agreement procedure is to enable mutual authentication between the UE and the network and to provide keying material that can be used between the UE and network in subsequent security procedures. The UE and the AMF shall support the EAP based primary authentication and key agreement procedure.

In order to initiate the EAP based primary authentication and key agreement procedure using EAP-AKA', the AUSF shall send an EAP message IE with EAP-request/AKA'-challenge message in the AUTHENTICATION REQUEST message.

The 5G NAS security context parameters from a full native 5G NAS security context shall be stored on the USIM if the corresponding file is present on the USIM as specified in TS 31.102 [19]. If the corresponding file is not present on the USIM, this 5GMM parameters is stored in a non-volatile memory in the ME together with the SUPI from the USIM.

The EF5GS3GPPNSC contains the 5GS 3GPP access NAS security context as defined in TS 24.501 [25], consisting of KAMF with the associated key set identifier, the UE security capabilities, and the uplink and downlink NAS COUNT values. This file shall contain one record.

The EF5GAUTHKEYS contains KAUSF and KSEAF that are generated on the ME using CK and IK as part of AKA procedures as described inTS 33.501 [24].

#### 15.1.1.2	Conformance requirement

CR 1	The UE shall support the EAP based primary authentication and key agreement procedure.

CR 2	The ME shall forward the RAND and AUTN received in EAP message IE with EAP-Request/AKA'-Challenge within the AUTHENTICATION REQUEST message to the USIM.

CR 3	The ME shall return the EAP message IE with EAP-response/AKA'-challenge in AUTHENTICATION RESPONSE message.

CR 4	As a result of successful authentication procedure and upon receipt of the EAP-Success message, the 5G NAS security context parameters shall be stored on the USIM if the corresponding file is present on the USIM when entering state 5GMM-DEREGISTERED.

CR 5	If service n°122 is available, the ME shall store KAMF with the associated key set identifier, the UE security capabilities, and the uplink and downlink NAS COUNT values in EF5GS3GPPNSC on the USIM.

CR 6	If service n°123 is available, the ME shall store the KAUSF and KSEAF in EF5GAUTHKEYS on the USIM.

CR 7	The UE can successfully register to the network.

Reference:

-	TS 31.102 [19], clauses 4.4.11.3, 4.4.11.4 and 4.4.11.6;

-	TS 33.501 [24], clause 6.1.3.1;

-	TS 24.501 [25], clause 5.4.1.2 and Annex C.

#### 15.1.1.3	Test purpose

The purpose of this test is to verify that:

1)	the primary authentication and key agreement procedure enables mutual authentication between the UE and the network and provides keying material that can be used between the UE and network in subsequent security procedures.

2)	the UE and the AMF support the EAP based primary authentication and key agreement procedure.

3)	the EAP based primary authentication and key agreement procedure using EAP-AKA' is performed if the AUSF has initiated an EAP message IE with EAP-request/AKA'-challenge message in the AUTHENTICATION REQUEST message.

4)	if the corresponding file is present on the USIM the 5G NAS security context parameters from a full native 5G NAS security context are stored on the USIM

5)	EF5GS3GPPNSC contains one record with the 5GS 3GPP access NAS security context as defined in TS 24.501 [42], consisting of KAMF with the associated key set identifier, the UE security capabilities, and the uplink and downlink NAS COUNT values.

6)	EF5GAUTHKEYS contains KAUSF and KSEAF that are generated on the ME using CK and IK as part of AKA procedures as described in TS 33.501 [41].

7)	the UE can successfully register to the network.

#### 15.1.1.4	Method of test

##### 15.1.1.4.1	Initial conditions

The values of the 5G-NR UICC as defined in clause 4.5.9 of the present document are used.

The TT (NG-SS) is configured to transmit on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):		244/083/000001

-	Access control:				unrestricted.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case.

##### 15.1.1.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | TT | Activate NG-SS cell | The NG-SS cell on the TT is activated with SIB1 as defined in the initial conditions |  |  |
| 2 | UE | Run initial activation |  |  |  |
| 3 | UE <> TT | Establish RRC connection | UE camps on NG-SS cell and establishes RRC connection |  |  |
| 4 | UE > TT | Send REGISTRATION REQUEST |  |  |  |
| 5 | TT > UE | Send AUTHENTICATION REQUEST | The TT sends an AUTHENTICATION REQUEST message with EAP message IE with EAP request/AKA' challenge message with: - ngKSI: - NAS key set identifier: '000' - TSC: '0' - EAP message: - EAP-request/AKA'-challenge The NG-SS on the TT generates KAMF, KAUSF and KSEAF values, and initiates the EAP-AKA' authentication procedure |  |  |
| 6 | ME > USIM | Pass RAND and AUTN values to the USIM | The ME provides the EAP request/AKA' challenge data received in the AUTHENTICATION REQUEST to the USIM | CR 1 CR 2 | A.2/1 OR A.2/2 |
| 7 | UE > TT | Send AUTHENTICATION RESPONSE | The UE sends an AUTHENTICATION RESPONSE message with EAP message IE with EAP response/AKA' challenge message: - Authentication response parameter: 4 – 16 octets RES value calculated according to TS 24.501 [25] The UE shall return RES equal to the XRES calculated at the NG-SS with the parameters provided/indicated in the AUTHENTICATION REQUEST at step 4 | CR 3 |  |
| 8 | TT > UE | Sends SECURITY MODE COMMAND message with EAP-success | The NG-SS sends a SECURITY MODE COMMAND message with EAP-success |  |  |
| 9 | UE > TT | Send SECURITY MODE COMPLETE | UE updates EF5GAUTHKEYS in parallel or after the NAS message | CR6 | A.2/1 OR A.2/2 |
| 10 | TT > UE | Send REGISTRATION ACCEPT | NG-SS sends a REGISTRATION ACCEPT message with: - 5G-GUTI: 24408300010266436587 - TAI: 244/83/000001 |  |  |
| 11 | UE > TT | UE sends REGISTRATION COMPLETE |  | CR7 |  |
| 12 | UE | The UE is powered off and/or deactivated, performs the DEREGISTRATION procedure. | UE enters the 5GMM-DEREGISTERED state and updates EF5GS3GPPNSC | CR4 | A.2/1 OR A.2/2 |

| Optional step if CR 4, CR 5 and CR 6 verification cannot be performed at step 9) and 12) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 13 | TT | READ EF5GS3GPPNSC and EF5GAUTHKEYS | The TT or the test operator shall perform whatever action is needed to get read access to EF5GS3GPPNSC and EF5GAUTHKEYS | CR 4 CR 5 CR 6 | A.2/3 |

#### 15.1.1.5	Acceptance criteria

CR 1 is verified if CR 2 is met, and the ME forwards the RAND and AUTN received in EAP-Request/AKA'-Challenge message to the USIM.

CR 3 requirements are met if the NG-SS receives an EAP message IE with EAP-response/AKA'-challenge in the AUTHENTICATION RESPONSE message sent in response to the AUTHENTICATION REQUEST in step 6).

CR 4 can be verified in step 12) or 13) after the state 5GMM-DEREGISTERED is entered and if an appropriate method to monitor the APDU for EF update or read the EF content is provided by the UE manufacturer.

CR 5 can be verified in step 12) or 13) if an appropriate method to monitor the APDU for EF update or read the EF content is provided by the UE manufacturer.

CR 6 can be verified in step 9) or 13) if an appropriate method to monitor the APDU for EF update or read the EF content is provided by the UE manufacturer.

CR 7 is verified if the UE is capable to register to the NG-SS at step 11).

The EF contents to be verified at step 9), 12) and/or 13) for CR 5 and 6 are listed as below:

EF5GAUTHKEYS (5G authentication keys)

Logically:

KAUSF:	32 bytes, value not checked

KSEAF:	32 bytes, value not checked

Coding:

| Byte | B1 | B2 | B3 | Bx | Bx+1 | Bx+2 | Bx+3 | ... | Bxx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 80 | L1 | Xx | .. | 81 | L2 | xx | … | xx |

EF5GS3GPPNSC (5GS 3GPP Access NAS Security Context)

Logically:

5GS NAS Security Context:

ngKSI:	00

KAMF:	32 bytes, value not checked

Uplink NAS count:	any value

Downlink NAS count:	any value

Identifiers of selected NAS integrity
and encryption algorithms:	any value

Identifiers of selected EPS NAS
integrity and encryption algorithms
for use after mobility to EPS:	any value

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | Bx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | A0 | XX | 80 | 01 | 00 | 81 | xx | xx | … | xx |

#### 15.1.1.5	Acceptance criteria

CR 1 is verified if CR 2 is met, and the ME forwards the RAND and AUTN received in EAP-Request/AKA'-Challenge message to the USIM.

CR 3 requirements are met if the NG-SS receives an EAP message IE with EAP-response/AKA'-challenge in the AUTHENTICATION RESPONSE message sent in response to the AUTHENTICATION REQUEST in step 6).

CR 4 can be verified in step 12) after the state 5GMM-DEREGISTERED is entered and if an appropriate method to read the EF content is provided by the UE manufacturer.

CR 5 and CR 6 can be verified in step 13) if an appropriate method to read the EF content is provided by the UE manufacturer.

CR 7 is verified if the UE is capable to register to the NG-SS at step 11).

### 15.1A.1	Authentication procedure for EAP-AKA' - Authentication is successful

#### 15.1A.1.1	Definition and applicability

The purpose of the primary authentication and key agreement procedure is to enable mutual authentication between the UE and the network and to provide keying material that can be used between the UE and network in subsequent security procedures. The UE and the AMF shall support the EAP based primary authentication and key agreement procedure.

In order to initiate the EAP based primary authentication and key agreement procedure using EAP-AKA', the AUSF shall send an EAP message IE with EAP-request/AKA'-challenge message in the AUTHENTICATION REQUEST message.

The 5G NAS security context parameters from a full native 5G NAS security context shall be stored on the USIM if the corresponding file is present on the USIM as specified in TS 31.102 [19]. If the corresponding file is not present on the USIM, this 5GMM parameters is stored in a non-volatile memory in the ME together with the SUPI from the USIM.

The EF5GS3GPPNSC contains the 5GS 3GPP access NAS security context as defined in TS 24.501 [25], consisting of KAMF with the associated key set identifier, the UE security capabilities, and the uplink and downlink NAS COUNT values. This file shall contain one record.

The EF5GSAUTHKEYS contains KAUSF and KSEAF that are generated on the ME using CK and IK as part of AKA procedures as described inTS 33.501 [24]. If service n°133 is "available" in EFUST, the EF5GAUTHKEYS also contains SOR counter and UE parameter update counter associated with the key KAUSF as described in 3GPP TS 33.501 [24] and 3GPP TS 31.102 [19].

#### 15.1A.1.2	Conformance requirement

CR 1	The UE shall support the EAP based primary authentication and key agreement procedure.

CR 2	The ME shall forward the RAND and AUTN received in EAP message IE with EAP-request/AKA'-challenge within the AUTHENTICATION REQUEST message to the USIM.

CR 3	The ME shall return the EAP message IE with EAP-response/AKA'-challenge in AUTHENTICATION RESPONSE message.

CR 4	As a result of successful authentication procedure and upon receipt of the EAP-Success message, the 5G NAS security context parameters shall be stored on the USIM if the corresponding file is present on the USIM when entering state 5GMM-DEREGISTERED.

CR 5	If service n°122 is available, the ME shall store KAMF with the associated key set identifier, the UE security capabilities, and the uplink and downlink NAS COUNT values in EF5GS3GPPNSC on the USIM.

CR 6	If service n°123 is available, and service n°133 is available the ME shall store the KAUSF, KSEAF, SOR counter and UE parameter update counter in the EF5GAUTHKEYS on the USIM.

CR 7	The UE can successfully register to the network.

Reference:

-	TS 31.102 [19], clauses 4.4.11.3, 4.4.11.4 and 4.4.11.6;

-	TS 33.501 [24], clause 6.1.3.1;

-	TS 24.501 [25], clause 5.4.1.2 and Annex C.

#### 15.1A.1.3	Test purpose

The purpose of this test is to verify that:

1)	the ME forwards the RAND and AUTN received in EAP-request/AKA'-challenge within EAP message IE to the USIM.

2)	the ME returns the EAP message IE with EAP-response/AKA'-challenge in AUTHENTICATION RESPONSE message indicating the response calculated in the USIM (RES).

3)	the ME stores 5G NAS security context parameters, consisting of KAMF with the associated key set identifier in EF5GS3GPPNSC on the USIM if service n°122 is "available".

4)	the ME stores the KAUSF, KSEAF, SOR counter and UE parameter update in the EF5GAUTHKEYS on the USIM if services n°123 and n°133 are available.

5)	the UE can successfully register to the network.

#### 15.1A.1.4	Method of test

##### 15.1A.1.4.1	Initial conditions

The values of the 5G-NR UICC – support of Rel-16 featurs as defined in clause 4.5.11 of the present document are used.

The TT (NG-SS) is configured to transmit on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):		244/083/000001

-	Access control:				unrestricted.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case.

##### 15.1A.1.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | TT | Activate NG-SS cell | The NG-SS cell on the TT is activated with SIB1 as defined in the initial conditions |  |  |
| 2 | UE | Run initial activation |  |  |  |
| 3 | UE <> TT | Establish RRC connection | UE camps on NG-SS cell and establishes RRC connection |  |  |
| 4 | UE > TT | Send REGISTRATION REQUEST |  |  |  |
| 5 | TT > UE | Send AUTHENTICATION REQUEST | The TT sends an AUTHENTICATION REQUEST message with EAP message IE with EAP request/AKA' challenge message with: - ngKSI: - NAS key set identifier: '000' - TSC: '0' - EAP message: - EAP-request/AKA'-challenge The NG-SS on the TT generates KAMF, KAUSF and KSEAF values, and initiates the EAP-AKA' authentication procedure |  |  |
| 6 | ME > USIM | Pass RAND and AUTN values to the USIM | The ME provides the EAP request/AKA' challenge data received in the AUTHENTICATION REQUEST to the USIM | CR 1 CR 2 | A.2/1 OR A.2/2 |
| 7 | UE > TT | Send AUTHENTICATION RESPONSE | The UE sends an AUTHENTICATION RESPONSE message with EAP message IE with EAP response/AKA' challenge message: - Authentication response parameter: message identity: ‘0101 0111’ 4 – 16 octets RES value calculated according to TS 24.501 [25] The UE shall return RES equal to the XRES calculated at the NG-SS with the parameters provided/indicated in the AUTHENTICATION REQUEST at step 4 | CR 3 |  |
| 8 | TT > UE | Sends SECURITY MODE COMMAND message with EAP-success | The NG-SS sends a SECURITY MODE COMMAND message with EAP-success |  |  |
| 9 | UE > TT | Send SECURITY MODE COMPLETE | UE updates EF5GAUTHKEYS in parallel or after the NAS message | CR 6 | A.2/1 OR A.2/2 |
| 10 | TT > UE | Send REGISTRATION ACCEPT | NG-SS sends a REGISTRATION ACCEPT message with: - 5G-GUTI: 24408300010266436587 - TAI: 244/83/000001 |  |  |
| 11 | UE > TT | UE sends REGISTRATION COMPLETE |  | CR 7 |  |
| 12 | UE | The UE is powered off and/or deactivated, performs the DEREGISTRATION procedure. | UE enters the 5GMM-DEREGISTERED state and updates EF5GS3GPPNSC | CR 4 | A.2/1 OR A.2/2 |

| Optional step if CR 4, CR 5 and CR 6 verification cannot be performed at step 9) and 12) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 13 | TT | READ EF5GS3GPPNSC and EF5GAUTHKEYS | The TT or the test operator shall perform whatever action is needed to get read access to EF5GS3GPPNSC and EF5GAUTHKEYS | CR 4 CR 5 CR 6 | A.2/3 |

#### 15.1A.1.5	Acceptance criteria

CR 1 is verified if CR 2 is met, and the ME forwards the RAND and AUTN received in EAP-Request/AKA'-Challenge message to the USIM.

CR 3 requirements are met if the NG-SS receives an EAP message IE with EAP-response/AKA'-challenge in the AUTHENTICATION RESPONSE message sent in response to the AUTHENTICATION REQUEST in step 6).

CR 4 can be verified in step 12) or 13) after the state 5GMM-DEREGISTERED is entered and if an appropriate method to monitor the APDU for EF update or read the EF content is provided by the UE manufacturer.

CR 5 can be verified in step 12) or 13) if an appropriate method to monitor the APDU for EF update or read the EF content is provided by the UE manufacturer.

CR 6 can be verified in step 9) or 13) if an appropriate method to monitor the APDU for EF update or read the EF content is provided by the UE manufacturer.

CR 7 is verified if the UE is capable to register to the NG-SS at step 11).

The EF contents to be verified at step 9), 12) and/or 13) for CR 5 and 6 are listed as below:

EF5GAUTHKEYS (5G authentication keys)

Logically:

KAUSF:	32 bytes, value not checked

KSEAF:	32 bytes, value not checked

KSEAF for non-3GPP access:	32 bytes, length and value not checked

SOR counter:	2 bytes, value not checked

UE parameter update counter:	2 bytes, value not checked

Coding:

| Byte | B1 | B2 | B3 | Bx | Bx+1 | Bx+2 | Bx+3 | ... | By |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 80 | L1 | xx | .. | 81 | L2 | xx | … | xx |
|  | By+1 | By+2 | By+3 | Bz | Bz+1 | Bz+2 | Bz+3 | .. | Bi+1 |
|  | 82 | L3 | xx | .. | 83 | L4 | xx | .. | 84 |
|  | Bi+2 | Bi+3 | Bj | Bxx |
|  | L5 | xx | .. | xx |

EF5GS3GPPNSC (5GS 3GPP Access NAS Security Context)

Logically:

5GS NAS Security Context:

ngKSI:					00

KAMF:						32 bytes, value not checked

Uplink NAS count:		any value

Downlink NAS count:	any value

Identifiers of selected NAS integrity
and encryption algorithms:		any value

Identifiers of selected EPS NAS
integrity and encryption algorithms
for use after mobility to EPS:	any value

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | Bx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | A0 | xx | 80 | 01 | 00 | 81 | xx | xx | … | xx |

### 15.1.2	Authentication procedure for EAP-AKA' – Authentication is successful - GSM UICC

#### 15.1.2.1	Definition and applicability

The purpose of the primary authentication and key agreement procedure is to enable mutual authentication between the UE and the network and to provide keying material that can be used between the UE and network in subsequent security procedures. The UE and the AMF shall support the EAP based primary authentication and key agreement procedure.

The 5G NAS security context parameters from a full native 5G NAS security context shall be stored on the USIM if the corresponding file is present on the USIM as specified in TS 31.102 [19]. If the corresponding file is not present on the USIM, this 5GMM parameters is stored in a non-volatile memory in the ME together with the SUPI from the USIM.

During the authentication procedure if the USIM computes a Kc (i.e. GPRS Kc) from CK and IK using conversion function c3 as described in TS 33.102 [53], and sends it to the ME, then the ME shall ignore such GPRS Kc and not store the GPRS Kc on USIM or in ME.

#### 15.1.2.2	Conformance requirement

CR 1	The ME shall ignore the GPRS Kc and not store the GPRS Kc on USIM if the USIM computes a Kc (i.e. GPRS Kc) from CK and IK using conversion function c3 as described in TS 33.102 [53].

CR 2	The ME shall ignore the GPRS Kc and not store the GPRS Kc in ME if the USIM computes a Kc (i.e. GPRS Kc) from CK and IK using conversion function c3 as described in TS 33.102 [53].

CR 3	If Service n°122 and Service n°123 are not available on the USIM, the 5GS 3GPP access NAS security context parameters and the 5G authentication keys shall be stored in the non-volatile memory of the ME.

Reference:

-	TS 33.501 [24], clause 6.1.3.1;

-	TS 24.501 [25], clause 5.4.1.2 and Annex C.

#### 15.1.2.3	Test purpose

The purpose of this test is to verify that:

- the ME will ignore the GPRS Kc and will not store the GPRS Kc on USIM or in ME when the USIM computes a Kc (i.e. GPRS Kc) from CK and IK.

- the ME stores the 5GS 3GPP access NAS security context parameters and the 5G authentication keys inside the ME non-volatile memory when Service n°122 and Service n°123 are not available on the USIM.

#### 15.1.2.4	Method of test

##### 15.1.2.4.1	Initial conditions

The values of the 5G-NR UICC as defined in clause 4.5.9 of the present document are used.

The TT (NG-SS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):	244/083/000001.

-	Access control:	unrestricted.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case.

##### 15.1.2.4.2	Procedure

| Step | Direction | Action | Comment | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | TT | NG-SS Cell is powered up | The NG-SS on the TT is activated |  |  |
|  | UE | Run initial activation |  |  |  |
| 2 | UE<>TT | UE camps on NG-SS Cell and establishes RRC connection |  |  |  |
| 3 | UE > TT | UE sends REGISTRATION REQUEST |  |  |  |
| 4 | TT > UE | NG-SS sends REGISTRATION REJECT (cause: #13 Roaming not allowed in this tracking area) and release RRC connection | UE shall delete 5G-GUTI, last visited registered TAI, TAI list and ngKSI. |  |  |
| 5 | UE | The UE is powered off and then on again |  |  |  |
| 6 | UE<>TT | UE camps on NG-SS Cell and establishes RRC connection |  |  |  |
| 7 | UE > TT | UE sends REGISTRATION REQUEST message with: - ngKSI: - NAS key set identifier: '111' | UE shall indicate in the REGISTRATION REQUEST that no key is available for the NAS key set identifier |  |  |
| 8 | TT > UE | NG-SS sends AUTHENTICATION REQUEST with EAP message IE with EAP request/AKA' challenge message with: - ngKSI: - NAS key set identifier: '000' - TSC: '0' - EAP message: - EAP-request/AKA'-challenge | The NG-SS on the TT generates KAMF, KAUSF and KSEAF values, and initiates the EAP-AKA' authentication procedure |  |  |
| 9 | UE | ME passes the RAND and AUTN values to the USIM | The ME provides the EAP request/AKA' challenge data received in the AUTHENTICATION REQUEST to the USIM; the USIM calculates the response parameter KC (i.e. GPRS Kc) from CK and IK using conversion function c3 |  |  |
| 10 | UE > TT | UE sends AUTHENTICATION RESPONSE message with EAP message IE with EAP response/AKA' challenge message: - Authentication response parameter: 16 octets RES value calculated according to TS 24.501 [25] | ME does not store GPRS Kc on the USIM through step 10 to 15 | CR 1 |  |
| 11 | TT > UE | NG-SS sends SECURITY MODE COMMAND message with EAP-success |  |  |  |
| 12 | UE > TT | UE send SECURITY MODE COMPLETE |  |  |  |
| 13 | TT > UE | NG-SS sends REGISTRATION ACCEPT message with: - 5G-GUTI: 24408300010266436587 - TAI: 42 34 80 00 00 01 |  |  |  |
| 14 | UE > TT | UE sends REGISTRATION COMPLETE |  |  |  |
| 15 | UE | The UE is powered off and/or deactivated, performs the DEREGISTRATION procedure |  |  |  |
| 16 | UE | Run initial activation and/or activated |  |  |  |
| 17 | UE<>TT | UE camps on NG-SS Cell and establishes RRC connection |  |  |  |
| 18 | UE > TT | UE sends REGISTRATION REQUEST message that is Security protected (indicated by Security header) with - ngKSI: - NAS key set identifier: '000' - TSC: '0' - 5GS mobile identity: 5G-GUTI type with value 24408300010266436587 |  | CR 1 CR 2 CR 3 |  |
| 19 | TT > UE | NG-SS sends REGISTRATION ACCEPT |  |  |  |
| 20 | UE > TT | UE sends REGISTRATION COMPLETE |  |  |  |

#### 15.1.2.5	Acceptance criteria

CR 1, CR 2 and CR 3 can be implicitly verified in step 18 when UE sends REGISTRATION REQUEST message that is Security protected and includes ngKSI and 5G-GUTI type 5GS mobile identity as specified in the test procedure.

Optionally, CR1 can be explicitly verified with methods A.2/1 or A.2/2 that ME does not store GPRS Kc on the USIM through steps 10 to 15.

### 15.1.3	Authentication procedure for EAP-AKA' – AUTN fails on the USIM

#### 15.1.3.1	Definition and applicability

The purpose of the primary authentication and key agreement procedure is to enable mutual authentication between the UE and the network and to provide keying material that can be used between the UE and network in subsequent security procedures. The UE and the AMF shall support the EAP based primary authentication and key agreement procedure.

At receipt of the RAND and AUTN, the USIM shall verify the freshness of the AV' by checking whether AUTN can be accepted as described in TS 33.102 [53]. If so, the USIM computes a response RES. The USIM shall return RES, CK, IK to the ME. The ME shall derive CK' and IK'. If the verification of the AUTN fails on the USIM, then the USIM and ME shall proceed as described in clause 6.1.3.3 in TS 33.501 [24].

When 5G AKA is used or when EAP-AKA’ is used, at the receipt of the RAND and AUTN, if the verification of the AUTN fails, then the USIM indicates to the ME the reason for failure and in the case of a synchronisation failure passes the AUTS parameter (see TS 33.102 [53]) to the ME.

If EAP-AKA’ is used: The ME shall send the EAP-Response/AKA-Synchronization-Failure include the AT_AUTS attribute (RFC 4187 [54]).

#### 15.1.3.2	Conformance requirement

CR 1	The ME shall forward the RAND and AUTN received in the EAP message IE with EAP-request/AKA'-challenge in the AUTHENTICATION REQUEST message to the USIM.

CR 2	If the verification of the AUTN fails on the USIM, then ME shall send AUTHENTICATION FAILURE indicates EAP-Response/AKA-Synchronization-Failure and includes the AUTS from USIM.

CR 3	If the verification of the AUTN success, the UE shall send AUTHENTICATION RESPONSE message contains the response calculated in the USIM (RES)

Reference:

-	TS 33.501 [24], clause 6.1.3.1 and 6.1.3.3;

-	TS 24.501 [25], clauses 5.4.1.2.2.1.

-	RFC 4187 [54], clause 9.6.

#### 15.1.3.3	Test purpose

1)	To verify that the ME forwards the RAND and AUTN received in EAP-request/AKA'-challenge message to the USIM.

2)	To verify that the ME returns the EAP-Response/AKA-Synchronization-Failure if the verification of AUTN failed on the USIM due to a synchronisation failure.

#### 15.1.3.4	Method of test

##### 15.1.3.4.1	Initial conditions

The NG-SS transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):	244/083/000001.

-	Access control:	unrestricted.

The default 5G-NR UICC is used and installed into the ME and the UE runs an initial activation.

##### 15.1.3.4.2	Procedure

| Step | Direction | Action | Comment | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | TT | NG-SS Cell is powered up | The NG-SS on the TT is activated |  |  |
| 2 | UE<>TT | UE camps on NG-SS Cell and establishes RRC connection |  |  |  |
| 3 | UE > TT | UE sends REGISTRATION REQUEST |  |  |  |
| 4 | TT > UE | NG-SS sends AUTHENTICATION REQUEST with EAP message IE with EAP request/AKA' challenge message with: - ngKSI: - NAS key set identifier: '000' - TSC: '0' - Authentication parameter AUTN: '1111 1111 1111 1111' - EAP message: - EAP-request/AKA'-challenge | AUTHENTICATION REQUEST with the AMF field in the IE "Authentication parameter AUTN" set to "AMFRESYNCH" value to trigger SQN re-synchronisation procedure in the USIM (see TS 34.108 [5], 8.1.2.2) |  |  |
| 5 | UE | ME passes the RAND and AUTN values to the USIM | The ME provides the EAP request/AKA' challenge data received in the AUTHENTICATION REQUEST at step 4 to the USIM | CR1 | A.2/1 OR A.2/2 |
| 6 | UE | The UICC returns AUTS parameter to the ME indicating that the verification of AUTN failed due to a synchronisation failure |  |  |  |
| 7 | UE > TT | UE sends AUTHENTICATION FAILURE message with: - 5GMM cause: '0001 0101' - Authentication failure parameter: AUTS value calculated according to TS 34.108, 8.1.2.2 [5] | AUTHENTICATION RESPONSE message including the EAP-response/AKA'-synchronization-failure message, with AUTS from USIM | CR2 |  |
| 8 | TT > UE | NG-SS sends AUTHENTICATION REQUEST with EAP message IE with EAP request/AKA' challenge message with: - ngKSI: - NAS key set identifier: '000' - TSC: '0' - EAP message: - EAP-request/AKA'-challenge |  |  |  |
| 9 | UE | ME passes the RAND and AUTN values to the USIM | The ME provides the EAP request/AKA' challenge data received in the AUTHENTICATION REQUEST at step 8 to the USIM | CR1 | A.2/1 OR A.2/2 |
| 10 | UE > TT | UE sends AUTHENTICATION RESPONSE message with EAP message IE with EAP response/AKA' challenge message: - Authentication response parameter: 4 – 16 octets RES value calculated according to TS 24.501 [25] | The UE shall return RES equal to the XRES calculated at the NG-SS with the parameters provided/indicated in the AUTHENTICATION REQUEST at step 8 | CR3 |  |
| 11 | TT > UE | NG-SS sends SECURITY MODE COMMAND message with EAP-success |  |  |  |
| 12 | UE > TT | UE send SECURITY MODE COMPLETE |  |  |  |
| 13 | TT > UE | NG-SS sends REGISTRATION ACCEPT message with: - 5G-GUTI: 24408300010266436587 - TAI: 42 34 80 00 00 01 |  |  |  |
| 14 | UE > TT | UE sends REGISTRATION COMPLETE |  |  |  |

#### 15.1.3.5	Acceptance criteria

CR 1 can be verified at step 5 and 9.

CR 2 can be verified at step 7 with AUTS value verified according to TS 34.108, 8.1.2.2.

CR 3 can be verified at step 10.

### 15.1.4	Authentication procedure for EAP-AKA' - after SUPI is changed

#### 15.1.4.1	Definition and applicability

The purpose of the primary authentication and key agreement procedure is to enable mutual authentication between the UE and the network and to provide keying material that can be used between the UE and network in subsequent security procedures. The UE and the AMF shall support the EAP based primary authentication and key agreement procedure.

The 5G NAS security context parameters from a full native 5G NAS security context shall be stored on the USIM if the corresponding file is present on the USIM as specified in TS 31.102 [19]. If the corresponding file is not present on the USIM, this 5GMM parameters is stored in a non-volatile memory in the ME together with the SUPI from the USIM. These 5GMM parameters can only be used if the SUPI from the USIM matches the SUPI stored in the non-volatile memory; else the UE shall delete the 5GMM parameters.

#### 15.1.4.2	Conformance requirement

CR 1	The UE shall support the EAP based primary authentication and key agreement procedure.

CR 2	The 5G NAS security context parameters shall be stored on the USIM or in a non-volatile memory in the ME together with the SUPI from the USIM.

CR 3	The UE shall delete the 5GMM parameters if the SUPI from the USIM does not match the SUPI stored in the non-volatile memory.

Reference:

-	TS 33.501 [24], clause 6.1.3.1;

-	TS 24.501 [25], clauses 5.4.1.2 and Annex C.

#### 15.1.4.3	Test purpose

1)	To verify that the UE generates the 5G NAS security context parameters and stores them inside the non-volatile memory in the ME together with the SUPI from the USIM.

2)	To verify that the UE rejects the SECURITY MODE COMMAND with the existing 5G NAS security context parameters if the SUPI is changed.

#### 15.1.4.4	Method of test

##### 15.1.4.4.1	Initial conditions

The values of the 5G-NR UICC as defined in clause 4.5.9 of the present document are used with the following exception:

EFUST (USIM Service Table)

Services defined in Releases higher than Rel-15 might be available.

Logically:

| Service n°122: |  | 5GS Mobility Management Information | not available |
| --- | --- | --- | --- |
| Service n°123: |  | 5G Security Parameters | not available |

Coding:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary: | xxxx xx1x | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxx1 xx1x | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | … | B16 | B17 |  |  |
|  | xxxx xxxx | xxxx xxxx | xx11 xxxx | ... | xxx0 100x | xxxx xxxx |  |  |

The TT (NG-SS) transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):		244/083/000001.

-	Access control:				unrestricted.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case.

##### 15.1.4.4.2	Procedure

| Step | Direction | Action | Comment | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | TT | NG-SS Cell is powered up | The NG-SS on the TT is activated |  |  |
|  | UE | Run initial activation with IMSI set as 246081685533963 |  |  |  |
| 2 | UE<>TT | UE camps on NG-SS Cell and establishes RRC connection |  |  |  |
| 3 | UE > TT | UE sends REGISTRATION REQUEST |  |  |  |
| 4 | TT > UE | NG-SS sends REGISTRATION REJECT (cause: #13 Roaming not allowed in this tracking area) and release RRC connection | UE shall delete 5G-GUTI, last visited registered TAI, TAI list and ngKSI. |  |  |
| 5 | UE | The UE is powered off |  |  |  |
|  | TT | NG-SS Cell is powered off |  |  |  |
| 6 | UE | Run an UE activation with default IMSI |  |  |  |
|  | TT | NG-SS Cell is powered on |  |  |  |
| 7 | UE<>TT | UE camps on NG-SS Cell and establishes RRC connection |  |  |  |
| 8 | UE > TT | UE sends REGISTRATION REQUEST message with: - ngKSI: - NAS key set identifier: '111' - 5GS mobile identity: SUCI of the UE | UE shall indicate in the REGISTRATION REQUEST that no key is available for the NAS key set identifier |  |  |
| 9 | TT > UE | NG-SS sends AUTHENTICATION REQUEST with EAP message IE with EAP request/AKA' challenge message with: - ngKSI: - NAS key set identifier: '000' - TSC: '0' - EAP message: - EAP-request/AKA'-challenge | The NG-SS on the TT generates KAMF, KAUSF and KSEAF values, and initiates the EAP-AKA' authentication procedure |  |  |
| 10 | UE > TT | UE sends AUTHENTICATION RESPONSE message with EAP message IE with EAP response/AKA' challenge message: - Authentication response parameter: 16 octets RES value calculated according to TS 24.501 [25] | The UE shall return RES equal to the XRES calculated at the NG-SS with the parameters provided/indicated in the AUTHENTICATION REQUEST at step 9 |  |  |
| 11 | TT > UE | NG-SS sends SECURITY MODE COMMAND message with EAP-success |  |  |  |
| 12 | UE > TT | UE send SECURITY MODE COMPLETE |  |  |  |
| 13 | TT > UE | NG-SS sends REGISTRATION ACCEPT message with: - 5G-GUTI: 24408300010266436587 - TAI: 42 34 80 00 00 01 |  |  |  |
| 14 | UE > TT | UE sends REGISTRATION COMPLETE |  | CR 1 |  |
| 15 | UE <> TT | The UE is powered off and, performs the DEREGISTRATION procedure |  |  |  |
| 16 | TT | NG-SS Cell is powered off |  |  |  |
| 17 | UE | Run initial activation, with IMSI set as 246081685533963 |  |  |  |
|  | TT | NG-SS Cell is powered on |  |  |  |
| 18 | UE<>TT | UE camps on NG-SS Cell and establishes RRC connection |  |  |  |
| 19 | UE > TT | UE sends REGISTRATION REQUEST message with: - ngKSI: - NAS key set identifier: '111' - 5GS mobile identity: SUCI of the UE | UE shall indicate within the REGISTRATION REQUEST for the NAS key set identifier that no key is available, and use SUCI as 5GS mobile identity IE | CR 3 |  |
| 20 | TT > UE | NG-SS sends AUTHENTICATION REQUEST with EAP message IE with EAP request/AKA' challenge message with: - ngKSI: - NAS key set identifier: '000' - TSC: '0' - EAP message: - EAP-request/AKA'-challenge |  |  |  |
| 21 | UE > TT | UE sends AUTHENTICATION RESPONSE message with EAP message IE with EAP response/AKA' challenge message: - Authentication response parameter: 16 octets RES value calculated according to TS 24.501 [25] | The UE shall return RES equal to the XRES calculated at the NG-SS with the parameters provided/indicated in the AUTHENTICATION REQUEST at step 20 |  |  |
| 22 | TT > UE | NG-SS sends SECURITY MODE COMMAND with: - ngKSI: The ngKSI of the partial native 5G security context assigned in the SECURITY MODE COMMAND message sent in step 11 |  |  |  |
| 23 | UE > TT | UE send SECURITY MODE REJECT with - 5GMM cause: '0001 1000' | Cause #24 - Security mode rejected, unspecified | CR 2 |  |
| 24 | UE | The UE is powered off |  |  |  |

#### 15.1.4.5	Acceptance criteria

CR 1 can be verified at step 14.

CR 3 can be implicitly verified at step 19 where REGISTRATION REQUEST includes NAS key set identifier that no key is available and use SUCI as 5GS mobile identity IE.

CR 2 can be verified at step 24.

## 15.2	Authentication procedure for 5G AKA

### 15.2.1	Authentication procedure for 5G AKA - Authentication is successful

#### 15.2.1.1	Definition and applicability

The purpose of the 5G AKA based primary authentication and key agreement procedure is to provide mutual authentication between the UE and the network and to agree on the keys KAUSF, KSEAF and KAMF. The UE and the AMF shall support the 5G AKA based primary authentication and key agreement procedure.

The 5G NAS security context parameters from a full native 5G NAS security context shall be stored on the USIM if the corresponding file is present on the USIM as specified in TS 31.102 [19]. If the corresponding file is not present on the USIM, this 5GMM parameters is stored in a non-volatile memory in the ME together with the SUPI from the USIM.

The EF5GS3GPPNSC contains the 5GS 3GPP access NAS security context as defined in TS 24.501 [25], consisting of KAMF with the associated key set identifier, the UE security capabilities, and the uplink and downlink NAS COUNT values. This file shall contain one record.

The EF5GAUTHKEYS contains KAUSF and KSEAF that are generated on the ME using CK and IK as part of AKA procedures as described in TS 33.501[24]

#### 15.2.1.2	Conformance requirement

CR 1	The UE shall support the 5G AKA based primary authentication and key agreement procedure.

CR 2	The ME shall forward the RAND and AUTN received in AUTHENTICATION REQUEST message to the USIM.

CR 3	The ME shall compute RES* from RES according to Annex A.4 TS 33.501[24] and return it in AUTHENTICATION RESPONSE message.

CR 4	If service n°122 is "available", when entering state 5GMM-DEREGISTERED, the ME shall store the KAMF with the associated key set identifier, the UE security capabilities, and the uplink and downlink NAS COUNT values in EF5GS3GPPNSC on the USIM.

CR 5	If service n°123 is "available", the ME shall store the KAUSF and KSEAF in EF5GAUTHKEYS on the USIM.

CR 6	The UE can successfully register to the network.

Reference:

-	TS 31.102 [19], clauses 4.4.11.3, 4.4.11.4 and 4.4.11.6;

-	TS 33.501 [24], clause 6.1.3.2;

-	TS 24.501 [25], clause 5.4.1.3 and Annex C.

#### 15.2.1.3	Test purpose

The purpose of this test is to verify that:1)	the ME forwards the RAND and AUTN received in 5G authentication challenge data within AUTHENTICATION REQUEST message to the USIM.

2)	the ME sends AUTHENTICATION RESPONSE message contains the calculated RES* in response to AUTHENTICATION REQUEST message.

3)	the ME stores 5G NAS security context parameters when entering state 5GMM-DEREGISTERED, consisting of KAMF with the associated key set identifier in EF5GS3GPPNSC on the USIM if service n°122 is "available".

4)	the ME stores the KAUSF and KSEAF in EF5GAUTHKEYS on the USIM if service n°123 is "available".

#### 15.2.1.4	Method of test

##### 15.2.1.4.1	Initial conditions

The NG-SS transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):	244/083/000001.

-	Access control:	unrestricted.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case.

##### 15.2.1.4.2	Procedure

| Step | Direction | Action | Comment | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | TT | NG-SS Cell is powered up with SIB1 as defined in the initial conditions | The NG-SS on the TT is activated |  |  |
|  | UE | Run initial activation |  |  |  |
| 2 | UE<>TT | UE camps on NG-SS Cell and establishes RRC connection |  |  |  |
| 3 | UE > TT | UE sends REGISTRATION REQUEST |  |  |  |
| 4 | TT > UE | NG-SS sends AUTHENTICATION REQUEST with 5G AKA-Challenge message: - ngKSI: - NAS key set identifier: '000' - TSC: '0' - Authentication parameter RAND (5G authentication challenge): 128 bits value - Authentication parameter AUTN (5G Authentication challenge). 128 bits value | The NG-SS on the TT generates KAMF, KAUSF and KSEAF values, and initiates the 5G AKA authentication procedure |  |  |
| 5 | UE | ME passes the RAND and AUTN values to the USIM | The ME provides the authentication challenge data received in the AUTHENTICATION REQUEST to the USIM | CR 1 CR 2 | A.2/1 OR A.2/2 |
| 6 | UE > TT | UE sends AUTHENTICATION RESPONSE message with: - Authentication response parameter: 16 octets RES* value calculated according to TS 24.501 [25] | The UE shall return RES* equal to the XRES* calculated at the NG-SS with the parameters provided/indicated in the AUTHENTICATION REQUEST | CR 3 |  |
| 7 | TT > UE | NG-SS sends SECURITY MODE COMMAND message including the ngKSI of the new 5G NAS security context (as provided in step 4) |  |  |  |
| 8 | UE > TT | UE send SECURITY MODE COMPLETE integrity protected and ciphered with the new 5G NAS security context identified by the ngKSI received in the SECURITY MODE COMMAND message in step 7 | UE updates EF5GAUTHKEYS in parallel or after the NAS message | CR 5 | A.2/1 OR A.2/2 OR A.2/3 |
| 9 | TT > UE | NG-SS sends REGISTRATION ACCEPT message with: - 5G-GUTI: 24408300010266436587 - TAI: 42 34 80 00 00 01 |  |  |  |
| 10 | UE > TT | UE sends REGISTRATION COMPLETE |  | CR 6 |  |
| 11 | UE <> TT | The UE is powered off and/or deactivated, performs the DEREGISTRATION procedure | UE enters the 5GMM-DEREGISTERED during this step, and updates EF5GS3GPPNSC | CR 4 | A.2/1 OR A.2/2 OR A.2/3 |
| 12 | TT | Read EF5GS3GPPNSC and EF5GAUTHKEYS | Optional step if CR4 and CR5 verification cannot be performed at step 8 and 11: The TT or the test operator perform whatever action is needed to get read access to EF5GS3GPPNSC and EF5GAUTHKEYS | CR 4CR 5 | A.2/3 |

#### 15.2.1.5	Acceptance criteria

CR 1 is verified if CR 2 is met, and the ME forwards the RAND and AUTN received in received in the AUTHENTICATION REQUEST to the USIM.

CR 3 requirements are met if the NG-SS receives RES* in the AUTHENTICATION RESPONSE message sent in response to the AUTHENTICATION REQUEST in step 6.

CR 4 can be verified in step 11 or 12 if an appropriate method to monitor the APDU for EF update or read the EF content is provided by the UE manufacturer.

CR 5 can be verified in step 8) or 12) if an appropriate method to monitor the APDU for EF update or read the EF content is provided by the UE manufacturer.

CR 6 is verified if the UE is capable to register to the NG-SS at step 13).

The EF contents to be verified at step 8), 11) and/or 12) for CR 4 and CR 5 are listed as below:

EF5GAUTHKEYS (5G authentication keys)

Logically:

KAUSF:	value not checked

KSEAF:	value not checked

Coding:

| Byte | B1 | B2 | B3 | Bx | Bx+1 | Bx+2 | Bx+3 | .. | Bxx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 80 | L1 | xx | .. | 81 | L2 | xx | .. | xx |

7)	After step 11) the ME updates EF5GS3GPPNSC as shown below.

EF5GS3GPPNSC (5GS 3GPP Access NAS Security Context)

Logically:

5GS NAS Security Context:

ngKSI:	00

KAMF:	value not checked

Uplink NAS count:	any value

Downlink NAS count:	any value

Identifiers of selected NAS integrity
and encryption algorithms:	any value

Identifiers of selected EPS NAS
integrity and encryption algorithms
for use after mobility to EPS:	any value

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | Bx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | A0 | xx | 80 | 01 | 00 | 81 | xx | xx | … | xx |

### 15.2A.1	Authentication procedure for 5G AKA - Authentication is successful

#### 15.2A.1.1	Definition and applicability

The purpose of the 5G AKA based primary authentication and key agreement procedure is to provide mutual authentication between the UE and the network and to agree on the keys KAUSF, KSEAF and KAMF. The UE and the AMF shall support the 5G AKA based primary authentication and key agreement procedure.

The 5G NAS security context parameters from a full native 5G NAS security context shall be stored on the USIM if the corresponding file is present on the USIM as specified in TS 31.102 [19]. If the corresponding file is not present on the USIM, this 5GMM parameters is stored in a non-volatile memory in the ME together with the SUPI from the USIM.

The EF5GS3GPPNSC contains the 5GS 3GPP access NAS security context as defined in TS 24.501 [25], consisting of KAMF with the associated key set identifier, the UE security capabilities, and the uplink and downlink NAS COUNT values. This file shall contain one record.

The EF5GAUTHKEYS contains KAUSF and KSEAF that are generated on the ME using CK and IK as part of AKA procedures as described in TS 33.501[24]

#### 15.2A.1.2	Conformance requirement

CR 1	The UE shall support the 5G AKA based primary authentication and key agreement procedure.

CR 2	The ME shall forward the RAND and AUTN received in AUTHENTICATION REQUEST message to the USIM.

CR 3	The ME shall compute RES* from RES according to Annex A.4 TS 33.501[24] and return it in AUTHENTICATION RESPONSE message.

CR 4	If service n°122 is "available", when entering state 5GMM-DEREGISTERED, the ME shall store the KAMF with the associated key set identifier, the UE security capabilities, and the uplink and downlink NAS COUNT values in EF5GS3GPPNSC on the USIM.

CR 5	If services n°123 and n°133 are available, the ME shall store the KAUSF, KSEAF, SOR counter and UE parameter update counter in EF5GAUTHKEYS on the USIM.

CR 6	The UE can successfully register to the network.

Reference:

-	TS 31.102 [19], clauses 4.4.11.3, 4.4.11.4 and 4.4.11.6;

-	TS 33.501 [24], clause 6.1.3.2;

-	TS 24.501 [25], clause 5.4.1.3 and Annex C.

#### 15.2A.1.3	Test purpose

The purpose of this test is to verify that:1)	the ME forwards the RAND and AUTN received in 5G authentication challenge data within AUTHENTICATION REQUEST message to the USIM.

2)	the ME sends AUTHENTICATION RESPONSE message contains the calculated RES* in response to AUTHENTICATION REQUEST message.

3)	the ME stores 5G NAS security context parameters when entering state 5GMM-DEREGISTERED, consisting of KAMF with the associated key set identifier in EF5GS3GPPNSC on the USIM if service n°122 is "available".

4)	the ME stores the KAUSF, KSEAF, SOR counter and UE parameter update counter in EF5GAUTHKEYS on the USIM if services n°123 and n°133 are available.

#### 15.2A.1.4	Method of test

##### 15.2A.1.4.1	Initial conditions

The values of the 5G-NR UICC – support of Rel-16 featurs as defined in clause 4.5.11 of the present document are used.

The TT (NG-SS) is configured to transmit on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):		244/083/000001

-	Access control:				unrestricted.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case.

##### 15.2A.1.4.2	Procedure

| Step | Direction | Action | Comment | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | TT | NG-SS Cell is powered up with SIB1 as defined in the initial conditions | The NG-SS on the TT is activated |  |  |
| 2 | UE | Run initial activation |  |  |  |
| 3 | UE<>TT | UE camps on NG-SS Cell and establishes RRC connection |  |  |  |
| 4 | UE > TT | UE sends REGISTRATION REQUEST |  |  |  |
| 5 | TT > UE | NG-SS sends AUTHENTICATION REQUEST with 5G AKA-Challenge message: - ngKSI: - NAS key set identifier: '000' - TSC: '0' - Authentication parameter RAND (5G authentication challenge): 128 bits value - Authentication parameter AUTN (5G Authentication challenge). 128 bits value | The NG-SS on the TT generates KAMF, KAUSF and KSEAF values, and initiates the 5G AKA authentication procedure |  |  |
| 6 | UE | ME passes the RAND and AUTN values to the USIM | The ME provides the authentication challenge data received in the AUTHENTICATION REQUEST to the USIM | CR 1 CR 2 | A.2/1 OR A.2/2 |
| 7 | UE > TT | UE sends AUTHENTICATION RESPONSE message with: - Authentication response parameter: 16 octets RES* value calculated according to TS 24.501 [25] | The UE shall return RES* equal to the XRES* calculated at the NG-SS with the parameters provided/indicated in the AUTHENTICATION REQUEST | CR 3 |  |
| 8 | TT > UE | NG-SS sends SECURITY MODE COMMAND message including the ngKSI of the new 5G NAS security context (as provided in step 4) |  |  |  |
| 9 | UE > TT | UE send SECURITY MODE COMPLETE integrity protected and ciphered with the new 5G NAS security context identified by the ngKSI received in the SECURITY MODE COMMAND message in step 7 | UE updates EF5GAUTHKEYS in parallel or after the NAS message | CR 5 | A.2/1 OR A.2/2 |
| 10 | TT > UE | NG-SS sends REGISTRATION ACCEPT message with: - 5G-GUTI: 24408300010266436587 - TAI: 42 34 80 00 00 01 |  |  |  |
| 11 | UE > TT | UE sends REGISTRATION COMPLETE |  | CR 6 |  |
| 12 | UE <> TT | The UE is powered off and/or deactivated, performs the DEREGISTRATION procedure | UE enters the 5GMM-DEREGISTERED during this step, and updates EF5GS3GPPNSC | CR 4 | A.2/1 OR A.2/2 |

| Optional step if CR 4, and CR 5 verification cannot be performed at step 9) and 12) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 13 | TT | READ EF5GS3GPPNSC and EF5GAUTHKEYS | The TT or the test operator shall perform whatever action is needed to get read access to EF5GS3GPPNSC and EF5GAUTHKEYS | CR 4 CR 5 | A.2/3 |

#### 15.2A.1.5	Acceptance criteria

CR 1 is verified if CR 2 is met, and the ME forwards the RAND and AUTN received in received in the AUTHENTICATION REQUEST to the USIM.

CR 3 requirements are met if the NG-SS receives RES* in the AUTHENTICATION RESPONSE message sent in response to the AUTHENTICATION REQUEST in step 6).

CR 4 can be verified in step 12) or 13) if an appropriate method to monitor the APDU for EF update or read the EF content is provided by the UE manufacturer.

CR 5 can be verified in step 9) or 13) if an appropriate method to monitor the APDU for EF update or read the EF content is provided by the UE manufacturer.

CR 6 is verified if the UE is capable to register to the NG-SS at step 11).

The EF contents to be verified at step 9), 12) and/or 13) for CR 4 and CR 5 are listed as below:

EF5GAUTHKEYS (5G authentication keys)

Logically:

KAUSF:	32 bytes, value not checked

KSEAF:	32 bytes, value not checked

KSEAF for non-3GPP access:	32 bytes, length and value not checked

SOR counter:	2 bytes, value not checked

UE parameter update counter:	2 bytes, value not checked

Coding:

| Byte | B1 | B2 | B3 | Bx | Bx+1 | Bx+2 | Bx+3 | ... | By |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 80 | L1 | xx | .. | 81 | L2 | xx | … | xx |
|  | By+1 | By+2 | By+3 | Bz | Bz+1 | Bz+2 | Bz+3 | .. | Bi+1 |
|  | 82 | L3 | xx | .. | 83 | L4 | xx | .. | 84 |
|  | Bi+2 | Bi+3 | Bj | Bxx |
|  | L5 | xx | .. | xx |

EF5GS3GPPNSC (5GS 3GPP Access NAS Security Context)

Logically:

5GS NAS Security Context:

ngKSI:					00

KAMF:						32 bytes, value not checked

Uplink NAS count:		any value

Downlink NAS count:	any value

Identifiers of selected NAS integrity
and encryption algorithms:		any value

Identifiers of selected EPS NAS
integrity and encryption algorithms
for use after mobility to EPS:	any value

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | Bx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | A0 | xx | 80 | 01 | 00 | 81 | xx | xx | … | xx |

### 15.2.2	Authentication procedure for 5G AKA – Authentication is successful - GSM UICC

#### 15.2.2.1	Definition and applicability

The purpose of the 5G AKA based primary authentication and key agreement procedure is to provide mutual authentication between the UE and the network and to agree on the keys KAUSF, KSEAF and KAMF. The UE and the AMF shall support the 5G AKA based primary authentication and key agreement procedure.

The 5G NAS security context parameters from a full native 5G NAS security context shall be stored on the USIM if the corresponding file is present on the USIM as specified in TS 31.102 [19]. If the corresponding file is not present on the USIM, this 5GMM parameters is stored in a non-volatile memory in the ME together with the SUPI from the USIM.

During the authentication procedure if the USIM computes a Kc (i.e. GPRS Kc) from CK and IK using conversion function c3 as described in TS 33.102 [53], and sends it to the ME, then the ME shall ignore such GPRS Kc and not store the GPRS Kc on USIM or in ME.

#### 15.2.2.2	Conformance requirement

CR 1	The ME shall ignore the GPRS Kc and not store the GPRS Kc on USIM if the USIM computes a Kc (i.e. GPRS Kc) from CK and IK using conversion function c3 as described in TS 33.102 [53].

CR 2	The ME shall ignore the GPRS Kc and not store the GPRS Kc in ME if the USIM computes a Kc (i.e. GPRS Kc) from CK and IK using conversion function c3 as described in TS 33.102 [53].

CR 3	If Service n°122 and Service n°123 are not available on the USIM, the 5GS 3GPP access NAS security context parameters and the 5G authentication keys shall be stored in the non-volatile memory of the ME.

Reference:

-	TS 33.501 [24], clause 6.1.3.1;

-	TS 24.501 [25], clause 5.4.1.2 and Annex C.

#### 15.2.2.3	Test purpose

1)	To verify that the ME will ignore the GPRS Kc and will not store the GPRS Kc on USIM or in ME when the USIM computes a Kc (i.e. GPRS Kc) from CK and IK.

2)	To verify that the ME stores the 5GS 3GPP access NAS security context parameters and the 5G authentication keys inside the ME non-volatile memory when Service n°122 and Service n°123 are not available on the USIM.

#### 15.2.2.4	Method of test

##### 15.2.2.4.1	Initial conditions

The values of the default UICC as defined in clause 4.5.2 of the present document are used.

The NR-SS transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):	244/083/000001.

-	Access control:	unrestricted.

The default UICC is used and installed into the ME.

##### 15.2.2.4.2	Procedure

| Step | Direction | Action | Comment | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | TT | NG-SS Cell is powered up | The NG-SS on the TT is activated |  |  |
| 2 | UE | Run initial activation |  |  |  |
| 3 | UE<>TT | UE camps on NG-SS Cell and establishes RRC connection |  |  |  |
| 4 | UE > TT | UE sends REGISTRATION REQUEST |  |  |  |
| 5 | TT > UE | NG-SS sends REGISTRATION REJECT (cause: #13 Roaming not allowed in this tracking area) and release RRC connection | UE shall delete 5G-GUTI, last visited registered TAI, TAI list and ngKSI. |  |  |
| 6 | UE | The UE is powered off and then on again |  |  |  |
| 7 | UE<>TT | UE camps on NG-SS Cell and establishes RRC connection |  |  |  |
| 8 | UE > TT | UE sends REGISTRATION REQUEST message with: - ngKSI: - NAS key set identifier: '111' | UE shall indicate in the REGISTRATION REQUEST that no key is available for the NAS key set identifier |  |  |
| 9 | TT > UE | NG-SS sends AUTHENTICATION REQUEST with 5G AKA-Challenge message: - ngKSI: - NAS key set identifier: '000' - TSC: '0' - Authentication parameter RAND (5G authentication challenge): 128 bits value - Authentication parameter AUTN (5G Authentication challenge). 128 bits value | The NG-SS on the TT generates KAMF, KAUSF and KSEAF values, and initiates the 5G AKA authentication procedure |  |  |
| 10 | UE | ME passes the RAND and AUTN values to the USIM | The ME provides the 5G AKA challenge data received in the AUTHENTICATION REQUEST to the USIM; the USIM calculates the response parameter KC (i.e. GPRS Kc) from CK and IK using conversion function c3 |  |  |
| 11 | UE > TT | UE sends AUTHENTICATION RESPONSE message with: - Authentication response parameter: 16 octets RES* value calculated according to TS 24.501 [25] | ME does not store GPRS Kc on the USIM through step 10 to 15 | CR 1 | O |
| 12 | TT > UE | NG-SS sends SECURITY MODE COMMAND including the ngKSI of the new 5G NAS security context (as provided in step 8) |  |  |  |
| 13 | UE > TT | UE send SECURITY MODE COMPLETE |  |  |  |
| 14 | TT > UE | NG-SS sends REGISTRATION ACCEPT message with: - 5G-GUTI: 24408300010266436587 - TAI: 42 34 80 00 00 01 |  |  |  |
| 15 | UE > TT | UE sends REGISTRATION COMPLETE |  |  |  |
| 16 | UE <> TT | The UE is powered off and/or deactivated, performs the DEREGISTRATION procedure |  |  |  |
| 17 | UE | Run initial activation and/or activated |  |  |  |
| 18 | UE<>TT | UE camps on NG-SS Cell and establishes RRC connection |  |  |  |
| 19 | UE > TT | UE sends REGISTRATION REQUEST message that is Security protected (indicated by Security header) with - ngKSI: - NAS key set identifier: '000' - TSC: '0' - 5GS mobile identity: 5G-GUTI type with value 24408300010266436587 |  | CR 1 CR 2 CR 3 |  |
| 20 | TT > UE | NG-SS sends REGISTRATION ACCEPT message with: - 5G-GUTI: 24408300010266436587 - TAI: 42 34 80 00 00 01 |  |  |  |
| 21 | UE > TT | UE sends REGISTRATION COMPLETE |  |  |  |

#### 15.2.2.5	Acceptance criteria

CR 1, CR 2 and CR 3 can be implicitly verified in step 19) when UE sends REGISTRATION REQUEST message that is Security protected and includes ngKSI and 5G-GUTI type 5GS mobile identity as specified in the test procedure.

Optionally, CR 1 can be explicitly verified with methods A.2/1 or A.2/2 that ME does not store GPRS Kc on the USIM through steps 11) to 16).

### 15.2A.2	Authentication procedure for 5G AKA - Authentication is successful

#### 15.2A.2.1	Definition and applicability

The purpose of the 5G AKA based primary authentication and key agreement procedure is to provide mutual authentication between the UE and the network and to agree on the keys KAUSF, KSEAF and KAMF. The UE and the AMF shall support the 5G AKA based primary authentication and key agreement procedure.

The 5G NAS security context parameters from a full native 5G NAS security context shall be stored on the USIM if the corresponding file is present on the USIM as specified in TS 31.102 [19]. If the corresponding file is not present on the USIM, this 5GMM parameters is stored in a non-volatile memory in the ME together with the SUPI from the USIM.

During the authentication procedure if the USIM computes a Kc (i.e. GPRS Kc) from CK and IK using conversion function c3 as described in TS 33.102 [53], and sends it to the ME, then the ME shall ignore such GPRS Kc and not store the GPRS Kc on USIM or in ME.

#### 15.2A.2.2	Conformance requirement

CR 1	The ME shall ignore the GPRS Kc and not store the GPRS Kc on USIM if the USIM computes a Kc (i.e. GPRS Kc) from CK and IK using conversion function c3 as described in TS 33.102 [53].

CR 2	The ME shall ignore the GPRS Kc and not store the GPRS Kc in ME if the USIM computes a Kc (i.e. GPRS Kc) from CK and IK using conversion function c3 as described in TS 33.102 [53].

CR 3	If Service n°122 and Service n°123 are not available on the USIM, the 5GS 3GPP access NAS security context parameters and the 5G authentication keys shall be stored in the non-volatile memory of the ME.

Reference:

-	TS 33.501 [24], clause 6.1.3.1;

-	TS 24.501 [25], clause 5.4.1.2 and Annex C.

#### 15.2A.2.3	Test purpose

The purpose of this test is to verify that:

1)	the ME will ignore the GPRS Kc and will not store the GPRS Kc on USIM or in ME when the USIM computes a Kc (i.e. GPRS Kc) from CK and IK.

2)	the ME stores the 5GS 3GPP access NAS security context parameters and the 5G authentication keys inside the ME non-volatile memory when Services n°122, n°123 and n°133 are not available on the USIM.

#### 15.2A.2.4	Method of test

##### 15.2A.2.4.1	Initial conditions

The values of the default UICC as defined in clause 4.5.2 of the present document are used.

The NR-SS transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):	244/083/000001.

-	Access control:	unrestricted.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case.

##### 15.2A.2.4.2	Procedure

| Step | Direction | Action | Comment | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | TT | NG-SS Cell is powered up | The NG-SS on the TT is activated |  |  |
| 2 | UE | Run initial activation |  |  |  |
| 3 | UE<>TT | UE camps on NG-SS Cell and establishes RRC connection |  |  |  |
| 4 | UE > TT | UE sends REGISTRATION REQUEST |  |  |  |
| 5 | TT > UE | NG-SS sends REGISTRATION REJECT (cause: #13 Roaming not allowed in this tracking area) and release RRC connection | UE shall delete 5G-GUTI, last visited registered TAI, TAI list and ngKSI. |  |  |
| 6 | UE | The UE is powered off and then on again |  |  |  |
| 7 | UE<>TT | UE camps on NG-SS Cell and establishes RRC connection |  |  |  |
| 8 | UE > TT | UE sends REGISTRATION REQUEST message with: - ngKSI: - NAS key set identifier: '111' | UE shall indicate in the REGISTRATION REQUEST that no key is available for the NAS key set identifier |  |  |
| 9 | TT > UE | NG-SS sends AUTHENTICATION REQUEST with 5G AKA-Challenge message: - ngKSI: - NAS key set identifier: '000' - TSC: '0' - Authentication parameter RAND (5G authentication challenge): 128 bits value - Authentication parameter AUTN (5G Authentication challenge). 128 bits value | The NG-SS on the TT generates KAMF, KAUSF and KSEAF values, and initiates the 5G AKA authentication procedure |  |  |
| 10 | UE | ME passes the RAND and AUTN values to the USIM | The ME provides the 5G AKA challenge data received in the AUTHENTICATION REQUEST to the USIM; the USIM calculates the response parameter KC (i.e. GPRS Kc) from CK and IK using conversion function c3 |  |  |
| 11 | UE > TT | UE sends AUTHENTICATION RESPONSE message with: - Authentication response parameter: 16 octets RES* value calculated according to TS 24.501 [25] | ME does not store GPRS Kc, SOR counter or UE parameter update counter on the USIM through step 11 to 16 | CR 1 |  |
| 12 | TT > UE | NG-SS sends SECURITY MODE COMMAND including the ngKSI of the new 5G NAS security context (as provided in step 8) |  |  |  |
| 13 | UE > TT | UE send SECURITY MODE COMPLETE |  |  |  |
| 14 | TT > UE | NG-SS sends REGISTRATION ACCEPT message with: - 5G-GUTI: 24408300010266436587 - TAI: 42 34 80 00 00 01 |  |  |  |
| 15 | UE > TT | UE sends REGISTRATION COMPLETE |  |  |  |
| 16 | UE <> TT | The UE is powered off and/or deactivated, performs the DEREGISTRATION procedure |  |  |  |
| 17 | UE | Run initial activation and/or activated |  |  |  |
| 18 | UE<>TT | UE camps on NG-SS Cell and establishes RRC connection |  |  |  |
| 19 | UE > TT | UE sends REGISTRATION REQUEST message that is Security protected (indicated by Security header) with - ngKSI: - NAS key set identifier: '000' - TSC: '0' - 5GS mobile identity: 5G-GUTI type with value 24408300010266436587 |  | CR 1 CR 2 CR 3 |  |
| 20 | TT > UE | NG-SS sends REGISTRATION ACCEPT message with: - 5G-GUTI: 24408300010266436587 - TAI: 42 34 80 00 00 01 |  |  |  |
| 21 | UE > TT | UE sends REGISTRATION COMPLETE |  |  |  |

#### 15.2A.2.5	Acceptance criteria

CR 1, CR 2 and CR 3 can be implicitly verified in step 19) when UE sends REGISTRATION REQUEST message that is security protected and includes ngKSI and 5G-GUTI type 5GS mobile identity as specified in the test procedure.

Optionally, CR 1 can be explicitly verified in steps 11) to 15) with methods A.2/1 or A.2/2 ensuring that the ME does not try to store GPRS Kc, SOR counter or UE parameter update counter on the USIM through steps 11) to 16).

### 15.2.3	Authentication procedure 5G AKA – AUTN fails on the USIM

#### 15.2.3.1	Definition and applicability

The purpose of the primary authentication and key agreement procedure is to enable mutual authentication between the UE and the network and to provide keying material that can be used between the UE and network in subsequent security procedures. The UE and the AMF shall support the EAP based primary authentication and key agreement procedure.

At receipt of the RAND and AUTN, the USIM shall verify the freshness of the AV' by checking whether AUTN can be accepted as described in TS 33.102 [53]. If so, the USIM computes a response RES. The USIM shall return RES, CK, IK to the ME.  If the verification of the AUTN fails on the USIM, then the USIM and ME shall proceed as described in clause 6.1.3. 3 in 3GPP TS 33.501 [24].

When 5G AKA is used or when EAP-AKA’ is used, at the receipt of the RAND and AUTN, if the verification of the AUTN fails, then the USIM indicates to the ME the reason for failure and in the case of a synchronisation failure passes the AUTS parameter (see TS 33.102 [53]) to the ME.

#### 15.2.3.2	Conformance requirement

CR 1	The ME shall forward the RAND and AUTN received in 5G authentication challenge data in the AUTHENTICATION REQUEST message to the USIM.

CR 2	If the verification of the AUTN fails on the USIM, then ME shall send AUTHENTICATION FAILURE with 5GMM cause #21 "Synch failure” and includes the AUTS from USIM.

CR 3	If the verification of the AUTN success, the UE shall send AUTHENTICATION RESPONSE message contains the response calculated in the USIM (RES)

Reference:

-	TS 33.501 [24], clause 6.1.3.2 and 6.1.3.3;

-	TS 24.501 [25], clause 5.4.1.3.7

#### 15.2.3.3	Test purpose

1)	To verify that the ME forwards the RAND and AUTN received in the 5G authentication challenge message to the USIM.

2)	To verify that the ME returns the Synchronization-Failure in the AUTHENTICATION FAILURE message if the verification of AUTN failed on the USIM due to a synchronisation failure.

#### 15.2.3.4	Method of test

##### 15.2.3.4.1	Initial conditions

The NG-SS transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):	244/083/000001.

-	Access control:	unrestricted.

The default 5G-NR UICC is used and installed into the ME.

##### 15.2.3.4.2	Procedure

| Step | Direction | Action | Comment | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | TT | NG-SS Cell is powered up | The NG-SS on the TT is activated |  |  |
|  | UE | Run initial activation |  |  |  |
| 2 | UE<>TT | UE camps on NG-SS Cell and establishes RRC connection |  |  |  |
| 3 | UE > TT | UE sends REGISTRATION REQUEST |  |  |  |
| 4 | TT > UE | NG-SS sends AUTHENTICATION REQUEST with 5G AKA challenge message with: - ngKSI: - NAS key set identifier: '000' - TSC: '0' - Authentication parameter AUTN: '1111 1111 1111 1111' - Authentication parameter RAND (5G authentication challenge): 128 bits value - Authentication parameter AUTN (5G Authentication challenge). 128 bits value | AUTHENTICATION REQUEST with the AMF field in the IE "Authentication parameter AUTN" set to "AMFRESYNCH" value to trigger SQN re-synchronisation procedure in the USIM (see TS 34.108 [5], 8.1.2.2) |  |  |
| 5 | UE | ME passes the RAND and AUTN values to the USIM | The ME provides the 5G AKA challenge data received in the AUTHENTICATION REQUEST at step 4 to the USIM | CR1 | A.2/1 OR A.2/2 |
| 6 | UE | The UICC returns AUTS parameter to the ME indicating that the verification of AUTN failed due to a synchronisation failure |  |  |  |
| 7 | UE > TT | UE sends AUTHENTICATION FAILURE message with: - 5GMM cause: '0001 0101' - Authentication failure parameter: AUTS value calculated according to TS 34.108, 8.1.2.2 [5] | AUTHENTICATION RESPONSE message including 5GMM cause "Synch failure", with AUTS from USIM | CR2 |  |
| 8 | TT > UE | NG-SS sends AUTHENTICATION REQUEST with 5G AKA challenge message with: - ngKSI: - NAS key set identifier: '000' - TSC: '0' - Authentication parameter RAND (5G authentication challenge): 128 bits value - Authentication parameter AUTN (5G Authentication challenge). 128 bits value |  |  |  |
| 9 | UE | ME passes the RAND and AUTN values to the USIM | The ME provides the 5G AKA challenge data received in the AUTHENTICATION REQUEST at step 8 to the USIM | CR1 | A.2/1 OR A.2/2 |
| 10 | UE > TT | UE sends AUTHENTICATION RESPONSE message with: - Authentication response parameter: 4 – 16 octets RES value calculated according to TS 24.501 [25] | The UE shall return RES* equal to the XRES* calculated at the NG-SS with the parameters provided/indicated in the AUTHENTICATION REQUEST at step 8 | CR3 |  |
| 11 | TT > UE | NG-SS sends SECURITY MODE COMMAND message including the ngKSI of the new 5G NAS security context (as provided in step 8) |  |  |  |
| 12 | UE > TT | UE send SECURITY MODE COMPLETE |  |  |  |
| 13 | TT > UE | NG-SS sends REGISTRATION ACCEPT message with: - 5G-GUTI: 24408300010266436587 - TAI: 42 34 80 00 00 01 |  |  |  |
| 14 | UE > TT | UE sends REGISTRATION COMPLETE |  |  |  |

#### 15.2.3.5	Acceptance criteria

CR 1 can be verified at step 5) and 9).

CR 2 can be verified at step 7) with AUTS value verified according to TS 34.108 [5]), 8.1.2.2.

CR 3 can be verified at step 10).

### 15.2.4	Authentication procedure for 5G AKA - after SUPI is changed

#### 15.2.4.1	Definition and applicability

The purpose of the 5G AKA based primary authentication and key agreement procedure is to provide mutual authentication between the UE and the network and to agree on the keys KAUSF, KSEAF and KAMF. The UE and the AMF shall support the 5G AKA based primary authentication and key agreement procedure.

The 5G NAS security context parameters from a full native 5G NAS security context shall be stored on the USIM if the corresponding file is present on the USIM as specified in TS 31.102 [19]. If the corresponding file is not present on the USIM, this 5GMM parameters is stored in a non-volatile memory in the ME together with the SUPI from the USIM. These 5GMM parameters can only be used if the SUPI from the USIM matches the SUPI stored in the non-volatile memory; else the UE shall delete the 5GMM parameters.

#### 15.2.4.2	Conformance requirement

CR 1	The UE shall support the 5G AKA based primary authentication and key agreement procedure.

CR 2	The 5G NAS security context parameters shall be stored on the USIM or in a non-volatile memory in the ME together with the SUPI from the USIM.

CR 3	The UE shall delete the 5GMM parameters if the SUPI from the USIM does not match the SUPI stored in the non-volatile memory.

Reference:

-	3GPP TS 33.501 [24], clause 6.1.3.2;

-	3GPP TS 24.501 [25], clauses 5.4.1.3, Annex C.

#### 15.2.4.3	Test purpose

1)	To verify that the UE generates the 5G NAS security context parameters and stores them inside the non-volatile memory in the ME together with the SUPI from the USIM.

2)	To verify that the UE rejects the SECURITY MODE COMMAND with the existing 5G NAS security context parameters if the SUPI is changed.

#### 15.2.4.4	Method of test

##### 15.2.4.4.1	Initial conditions

The NG-SS transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):	244/083/000001.

-	Access control:			unrestricted.

The 5G-NR UICC as defined in clause 4.5.9 is used with the following exceptions and installed into the ME.

EFUST (USIM Service Table)

Logically:

| Service n°122 |  | 5GS Mobility Management Information | not available |
| --- | --- | --- | --- |
| Service n°123: |  | 5G Security Parameters | not available |

Coding:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary: | xxxx xx1x | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxx1 xx1x | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | … | B16 | B17 |  |  |
|  | xxxx xxxx | xxxx xxxx | xx11 xxxx | ... | xxx0 100x | 0000 0xxx |  |  |

##### 15.2.4.4.2	Procedure

| Step | Direction | Action | Comment | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | TT | NG-SS Cell is powered up | The NG-SS on the TT is activated |  |  |
|  | UE | Run initial activation with IMSI set as 246081685533963 |  |  |  |
| 2 | UE<>TT | UE camps on NG-SS Cell and establishes RRC connection |  |  |  |
| 3 | UE > TT | UE sends REGISTRATION REQUEST |  |  |  |
| 4 | TT > UE | NG-SS sends REGISTRATION REJECT (cause: #13 Roaming not allowed in this tracking area) and release RRC connection | UE shall delete 5G-GUTI, last visited registered TAI, TAI list and ngKSI. |  |  |
| 5 | UE | The UE is powered off |  |  |  |
|  | TT | NG-SS Cell is powered off |  |  |  |
| 6 | UE | Run initial activation, with default IMSI |  |  |  |
|  | TT | NG-SS Cell is powered on |  |  |  |
| 7 | UE<>TT | UE camps on NG-SS Cell and establishes RRC connection |  |  |  |
| 8 | UE > TT | UE sends REGISTRATION REQUEST message with: - ngKSI: - NAS key set identifier: '111' - 5GS mobile identity: SUCI of the UE | UE shall indicate in the REGISTRATION REQUEST that no key is available for the NAS key set identifier |  |  |
| 9 | TT > UE | NG-SS sends AUTHENTICATION REQUEST with 5G AKA challenge message with: - ngKSI: - NAS key set identifier: '000' - TSC: '0' - Authentication parameter RAND (5G authentication challenge): 128 bits value - Authentication parameter AUTN (5G Authentication challenge). 128 bits value | The NG-SS on the TT generates KAMF, KAUSF and KSEAF values, and initiates 5G AKA authentication procedure |  |  |
| 10 | UE > TT | UE sends AUTHENTICATION RESPONSE message with: - Authentication response parameter: 16 octets RES* value calculated according to TS 24.501 [25] | The UE shall return RES* equal to the XRES* calculated at the NG-SS with the parameters provided/indicated in the AUTHENTICATION REQUEST at step 9 |  |  |
| 11 | TT > UE | NG-SS sends SECURITY MODE COMMAND message including the ngKSI of the new 5G NAS security context (as provided in step 9) |  |  |  |
| 12 | UE > TT | UE send SECURITY MODE COMPLETE |  |  |  |
| 13 | TT > UE | NG-SS sends REGISTRATION ACCEPT message with: - 5G-GUTI: 24408300010266436587 - TAI: 42 34 80 00 00 01 |  |  |  |
| 14 | UE > TT | UE sends REGISTRATION COMPLETE |  | CR1 |  |
| 15 | UE <> TT | The UE is powered off and, performs the DEREGISTRATION procedure |  |  |  |
| 16 | TT | NG-SS Cell is powered off |  |  |  |
| 17 | UE | Run initial activation, with IMSI set as 246081685533963 |  |  |  |
|  | TT | NG-SS Cell is powered on |  |  |  |
| 18 | UE<>TT | UE camps on NG-SS Cell and establishes RRC connection |  |  |  |
| 19 | UE > TT | UE sends REGISTRATION REQUEST message with: - ngKSI: - NAS key set identifier: '111' - 5GS mobile identity: SUCI of the UE | UE shall indicate within the REGISTRATION REQUEST for the NAS key set identifier that no key is available, and use SUCI as 5GS mobile identity IE | CR3 |  |
| 20 | TT > UE | NG-SS sends AUTHENTICATION REQUEST with 5G AKA challenge message with: - ngKSI: - NAS key set identifier: '000' - TSC: '0' - Authentication parameter RAND (5G authentication challenge): 128 bits value - Authentication parameter AUTN (5G Authentication challenge). 128 bits value |  |  |  |
| 21 | UE > TT | UE sends AUTHENTICATION RESPONSE message with: - Authentication response parameter: 16 octets RES* value calculated according to TS 24.501 [25] | The UE shall return RES* equal to the XRES* calculated at the NG-SS with the parameters provided/indicated in the AUTHENTICATION REQUEST at step 20 |  |  |
| 22 | TT > UE | NG-SS sends SECURITY MODE COMMAND with: - ngKSI: The ngKSI of the partial native 5G security context assigned in the SECURITY MODE COMMAND message sent in step 11 |  |  |  |
| 23 | UE > TT | UE send SECURITY MODE REJECT with - 5GMM cause: '0001 1000' | Cause #24 - Security mode rejected, unspecified | CR2 |  |
| 24 | UE | The UE is powered off |  |  |  |

#### 15.2.4.5	Acceptance criteria

CR 1 can be verified at step 14.

CR 3 can be implicitly verified at step 19 where REGISTRATION REQUEST includes NAS key set identifier that no key is available and use SUCI as 5GS mobile identity IE.

CR 2 can be verified at step 24.

### 15.2.5	Authentication procedure for registration on 3GPP accesses. successful 5G AKA - Authentication in different PLMNs

#### 15.2.5.1	Definition and applicability

The purpose of the 5G AKA based primary authentication and key agreement procedure is to provide mutual authentication between the UE and the network and to agree on the keys KAUSF, KSEAF and KAMF. The UE and the AMF shall support the 5G AKA based primary authentication and key agreement procedure.

If the UE supports multiple records of NAS security context storage for multiple registration, the UE shall store the previously current 5G NAS security context in the second 5G security context of that access (see 3GPP TS 31.102 [19]) and store the new 5G security context in the first 5G security context, when the UE activates the new 5G security context for a different PLMN over that access but the previously current 5G NAS security context is associated with the 5G-GUTI of the other access.

If the previously current 5G NAS security context is not associated with the 5G-GUTI of the other access (e.g. UE uses only 3GPP access) the second record will not be used.

The EF5GS3GPPNSC contains the 5GS 3GPP access NAS security context as defined in 3GPP TS 24.501 [25], consisting of KAMF with the associated key set identifier, the UE security capabilities, and the uplink and downlink NAS COUNT values. This file shall contain two records if service n°136 is "available".

The EF5GAUTHKEYS contains KAUSF and KSEAF that are generated on the ME using CK and IK as part of AKA procedures as described in 3GPP TS 33.501 [24]. If service n°133 is "available" in EFUST, the EF5GAUTHKEYS also contains SOR counter and UE parameter update counter associated with the key KAUSF as described in 3GPP TS 33.501 [24] and 3GPP TS 31.102 [19].

#### 15.2.5.2	Conformance requirement

CR 1	The UE shall support the 5G AKA based primary authentication and key agreement procedure

Reference:

-	3GPP TS 24.501 [25], clauses 4.4.2, 5.4.1.1 and 5.4.1.3

CR 2	The ME shall forward the RAND and AUTN received in AUTHENTICATION REQUEST message to the USIM.

Reference:

-	3GPP TS 24.501 [25], clause 5.4.1.3.6

CR 3	The ME shall compute RES* from RES according to TS 33.501 [24], Annex A.4 and return it in the AUTHENTICATION RESPONSE message.

Reference:

-	3GPP TS 33.501 [24], Annex A.4

CR 4	As a result of the first successful authentication procedure in a PLMN (PLMN 1 over 3GPP access), the 5G 3GPP access NAS security context parameters shall be stored on the USIM in the first record of the EF5GS3GPPNSC.

Reference:

-	3GPP TS 31.102 [19], clauses 4.4.11.4 and 5.2.32

CR 5	As a result of the second successful authentication procedure in another PLMN (PLMN 2 over 3GPP access), the 5G 3GPP access NAS security context parameters of PLMN 2 shall be stored on the USIM in the first record of the EF5GS3GPPNSC as the previously current 5G NAS security context is not associated with the 5G-GUTI of the other access.

Reference:

-	3GPP TS 31.102 [19], clauses 4.4.11.4 and 5.2.32

CR 6	The 5G 3GPP access NAS security context parameters of PLMN 1 will be overwritten with the 5G 3GPP access NAS security context parameters of PLMN 2. The second record of the EF5GS3GPPNSC is not used.

Reference:

-	3GPP TS 31.102 [19], clauses 4.4.11.4 and 5.2.32

#### 15.2.5.3	Test purpose

The purpose of this test is to verify that:

1)	the ME forwards the RAND and AUTN received in 5G authentication challenge data from the first PLMN to the USIM within the AUTHENTICATION REQUEST message;

2)	the ME sends AUTHENTICATION RESPONSE message contains the calculated RES* in response to AUTHENTICATION REQUEST message;

3)	the ME stores 5G 3GPP access NAS security context parameters when entering state 5GMM-DEREGISTERED, consisting of KAMF with the associated key set identifier in the first record of thr EF5GS3GPPNSC on the USIM;

4)	the ME forwards the RAND and AUTN received in 5G authentication challenge data from the second PLMN to the USIM within the AUTHENTICATION REQUEST message;

5)	the ME sends AUTHENTICATION RESPONSE message contains the calculated RES* in response to AUTHENTICATION REQUEST message;

6)	the ME stores 5G 3GPP access NAS security context parameters of a different PLMN when entering state 5GMM-DEREGISTERED, consisting of KAMF with the associated key set identifier in the first record of the EF5GS3GPPNSC on the USIM.

#### 15.2.5.4	Method of test

##### 15.2.5.4.1	Initial conditions

The 5G-NR UICC - support of Rel-16 features as defined in clause 4.5.11 of the present document is used.

The TT (NG-SS) transmits on the BCCH, with the following network parameters:

Cell A:

TAI (MCC/MNC/TAC):		244/083/000001

CellIdentity:					"000000001"

Access control:				unrestricted.

Cell B :

TAI (MCC/MNC/TAC):		244/084/000001

CellIdentity:					"000000001"

Access control:				unrestricted.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case.

##### 15.2.5.4.2	Procedure

| Step | Direction | Action | Comment | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | TT | Power up TT (NG-SS) Cell A is with SIB1 as defined in the initial conditions | The NG-SS on the TT is activated |  |  |
| 2 | UE | Run initial activation | The ME reads EFUST, EF5GS3GPPNSC and EF5GAUTHKEYS | CR 1 | A.2/1 OR A.2/2 |
| 3 | UE > TT | Send RRCSetupRequest message | TT (NG-SS) transmits an RRCSetup message to the UE |  |  |
| 4 | UE > TT | Send RRCSetupComplete message |  |  |  |
| 5 | UE > TT | Send REGISTRATION REQUEST | Within the REGISTRATION REQUEST the UE indicates that no key is available. |  |  |
| 6 | TT > UE | Send AUTHENTICATION REQUEST with 5G AKA-Challenge message: - ngKSI: - NAS key set identifier: '000' - TSC: '0' - Authentication parameter RAND (5G authentication challenge): 128 bits value - Authentication parameter AUTN (5G Authentication challenge). 128 bits value | TT (NG-SS) generates KAMF, KAUSF and KSEAF values, and initiates the 5G AKA authentication procedure | CR 1 |  |
| 7 | UE | Pass RAND and AUTN values to the USIM | The ME provides the authentication challenge data received in the AUTHENTICATION REQUEST to the USIM | CR 2 | A.2/1 OR A.2/2 |
| 8 | UE <> TT | Send AUTHENTICATION RESPONSE message with: - Authentication response parameter: 16 octets RES* value | The TT (NG-SS) sends a SECURITY MODE COMMAND message | CR 3 |  |
| 9 | UE > TT | Send SECURITY MODE COMPLETE message |  |  |  |
| 10 | TT > UE | Send REGISTRATION ACCEPT message with: - 5G-GUTI: 24408300010266436587 - TAI: 42 34 80 00 00 01 |  |  |  |
| 11 | UE > TT | Send REGISTRATION COMPLETE |  |  |  |
| 12 | UE > TT/USIM | The UE is powered off or deactivated, and performs the deregistration procedure | UE enters the 5GMM-DEREGISTERED during this step, and updates EF5GS3GPPNSC | CR 4 | A.2/1 OR A.2/2 |
| 13 | TT | Power down TT (NG-SS) Cell A Power up TT (NG-SS) Cell B with SIB1 as defined in the initial conditions |  |  |  |
| 14 | UE | Run activation or switch on the UE’s radio interface | The UE reads the first record of the 5G 3GPP NAS security context from the USIM |  |  |
| 15 | UE > TT | Send RRCSetupRequest message | TT (NG-SS) transmits an RRCSetup message to the UE |  |  |
| 16 | UE > TT | Send RRCSetupComplete message |  |  |  |
| 17 | UE > TT | Send REGISTRATION REQUEST | TT (NG-SS) initiates the 5G AKA authentication procedure |  |  |
| 18 | TT > UE | Send AUTHENTICATION REQUEST with 5G AKA-Challenge message: - ngKSI: - NAS key set identifier: '001' - TSC: '0' - Authentication parameter RAND (5G authentication challenge): 128 bits value - Authentication parameter AUTN (5G Authentication challenge). 128 bits value | The NG-SS on the TT generates KAMF, KAUSF and KSEAF values, and initiates the 5G AKA authentication procedure |  |  |
| 19 | UE | Pass RAND and AUTN values to the USIM | The ME provides the authentication challenge data received in the AUTHENTICATION REQUEST to the USIM | CR 5 | A.2/1 OR A.2/2 |
| 20 | UE <> TT | Send AUTHENTICATION RESPONSE message | The TT (NG-SS) sends a SECURITY MODE COMMAND message |  |  |
| 21 | UE > TT | Send SECURITY MODE COMPLETE message |  |  |  |
| 22 | TT > UE | Send REGISTRATION ACCEPT message with: - 5G-GUTI: 24408300010266436587 - TAI: 42 34 80 00 00 01 |  |  |  |
| 23 | UE > TT | Send REGISTRATION COMPLETE message |  |  |  |
| 24 | UE > TT/USIM | Power off and/or deactivate the UE | UE enters the 5GMM-DEREGISTERED during this step, and updates EF5GS3GPPNSC | CR 6 | A.2/1 OR A.2/2 |

#### 15.2.5.5	Acceptance criteria

CR 1	can be explicitly verified by using methods A.2/1 or A.2/2 to check EFUST, EF5GS3GPPNSC and EF5GAUTHKEYS contents in step 2). With having sent the expected 5G AKA-Challenge message data within the AUTHENTICATION REQUEST in step 6) CR 1 is implicitly verified.

CR 2 can be implicitly verified in step 7) using methods A.2/1 or A.2/2. CR 2 is met if EF5GS3GPPNSC has the expected contents:

EF5GS3GPPNSC (5GS 3GPP Access NAS Security Context)

Logically:

5GS NAS Security Context:

ngKSI:							00

KAMF:								32 bytes, value not checked

Uplink NAS count:				any value

Downlink NAS count:				any value

Identifiers of selected NAS integrity
and encryption algorithms:		any value

Identifiers of selected EPS NAS
integrity and encryption algorithms
for use after mobility to EPS:	any value

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | Bx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | A0 | xx | 80 | 01 | 00 | 81 | xx | xx | … | xx |

CR 3 is implicitly verified in step 8). The requirement is met if the ME sends a SECURITY MODE COMPLETE message in step 9).

CR 4 is explicitly verified in step 12) using methods A.2/1 or A.2/2. The requirement is met if EF5GS3GPPNSC has the expected contents:

EF5GS3GPPNSC (5GS 3GPP Access NAS Security Context)

Logically:

5GS NAS Security Context first record:

ngKSI:								00

KAMF:									value not checked

Uplink NAS count:					any value

Downlink NAS count:				any value

Identifiers of selected NAS integrity
and encryption algorithms:		any value

Identifiers of selected EPS NAS
integrity and encryption algorithms
for use after mobility to EPS:	any value

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | Bx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | A0 | xx | 80 | 01 | 00 | 81 | xx | xx | … | xx |

CR 5 can be explicitly verified in step 19) using methods A.2/1 or A.2/2. The requirement is met if EF5GAUTHKEYS has the expected contents:

EF5GAUTHKEYS (5G authentication keys)

Logically:

KAUSF:								32 bytes, value not checked

KSEAF for 3GPP access:			32 bytes, value not checked

KSEAF for non-3GPP access:		32 bytes, all values set to 'FF'

SOR counter:						2 bytes, value not checked

UE parameter update counter:	2 bytes, value not checked

Coding:

| Byte | B1 | B2 | B3 | .. | B35 | B36 | B37 | .. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 80 | 20 | xx | xx | 81 | 20 | xx | xx |
|  | B69 | B70 | B71 | .. | B103 | B104 | B105 | B106 |
|  | 82 | 20 | FF | FF | 83 | 02 | xx | xx |
|  | B107 | B108 | B109 | B110 |
|  | 84 | 02 | xx | xx |

CR 6 is explicitly verified if step 24) using methods A.2/1 or A.2/2. The requirement is met if EF5GS3GPPNSC has the expected contents:

EF5GS3GPPNSC (5GS 3GPP Access NAS Security Context)

Logically:

5GS NAS Security Context first record

ngKSI:								01

KAMF:									value not checked

Uplink NAS count:					any value

Downlink NAS count:				any value

Identifiers of selected NAS integrity
and encryption algorithms:		any value

Identifiers of selected EPS NAS
integrity and encryption algorithms
for use after mobility to EPS:	any value

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | Bx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | A0 | xx | 80 | 01 | 01 | 81 | xx | xx | … | xx |

5GS NAS Security Context second record:

ngKSI:								07 (no key available)

KAMF:									value not checked

Uplink NAS count:					any value

Downlink NAS count:				any value

Identifiers of selected NAS integrity
and encryption algorithms:		any value

Identifiers of selected EPS NAS
integrity and encryption algorithms
for use after mobility to EPS:	any value

PLMN identifier:					any value

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | Bx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | A0 | xx | 80 | 01 | 07 | 81 | xx | xx | … | xx |

NOTE:	Implicit verification using method A.2/3 is not possible for CR 2 and CR 4. Thus, A.2/3 is no valid verification method for this test.

# 16	UE Route Selection Policy (URSP) procedure

## 16.1	Pre-configured URSP rules

### 16.1.1	Support for URSP by USIM

#### 16.1.1.1	Definition and applicability

As specified in TS 24.526 [38], if the UE has no signalled URSP, and the UE has pre-configured URSPs configured in both the USIM and the ME, then the UE shall use the pre-configured URSP in the USIM.

As specified in TS 31.102 [19], the EFURSP in the USIM contains UE Route Selection Policies per PLMN and shall be considered by the ME if EFUST service n° 132 "Support for URSP by USIM" is available.

#### 16.1.1.2	Conformance requirement

CR 1	The UE shall support the URSP procedure;

Reference:

-	TS 31.102 [19], clause 5.2.34

-	TS 24.526 [38], clause 4.2.2.2

-	TS 23.503 [40], clause 6.6.2.1

-	TS 23.501 [39], clause 5.15.5.2 and 5.15.5.3

CR 2	The URSP rules shall be read from USIM if service n°132 is available;

Reference:

-	TS 31.102 [19], clause 4.4.11.12 and 5.2.3.34

CR 3	The ME shall use URSP rules stored on the USIM rather than any pre-configured USRP rules stored in the ME if service n°132 is available.

Reference:

-	TS 31.102 [19], clause 5.1.1 and 5.2.34

#### 16.1.1.3	Test purpose

The purpose of this test is to verify that:

1)	To verify that the ME reads the URSP rules from USIM if service n°132 is available.

2)	To verify that the ME uses the matching URSP rule from USIM to set the PDU session establishment parameters if service n°132 is available.

#### 16.1.1.4	Method of test

##### 16.1.1.4.1	Initial conditions

The values of the 5G-NR UICC – support of Rel-16 features as defined in clause 4.5.11 of the present document are used with EFIMSI as defined in clause 4.6.5 and the following exceptions:

EFUST (USIM Service Table)

Logically:

| Service n°132: |  | Support for URSP by USIM | available |
| --- | --- | --- | --- |

Coding:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary: | xxxx xx1x | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxx1 xx1x | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | … | B16 | B17 |  |  |
|  | xxxx xxxx | xxxx xxxx | xx11 xxxx | ... | xxx0 111x | 0xxx 1xxx |  |  |

EFURSP (UE Route Selection Policies)

Logically:

URSP rules for one PLMN only

-	PLMN:	246 081

Rule Precedence = 0

Traffic descriptor:

-	DNN = TestGp.rs

Route Selection Descriptor:

-	Precedence = 0

-	Network Slice Selection, S-NSSAI: '01 01 01 03' (ST: MBB, SD: 010103)

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 80 | 22 | 42 | 16 | 80 | 1E | 00 | 1C | 00 | 00 | 0C | 88 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 | 00 |
|  | B25 | B26 | B27 | B28 | B29 | B30 | B31 | B32 | B33 | B34 | B35 | B36 |
|  | 0B | 00 | 09 | 00 | 00 | 06 | 02 | 04 | 01 | 01 | 01 | 03 |

The ME is pre-configured with the following URSP rules:

USRP rules for one PLMN only

-	PLMN: 246 081

Rule Precedence =1

Traffic descriptor:

-	DNN=TestGp.rs

Route Selection Descriptor:

-	Precedence=1

-	Network Slice Selection, S-NSSAI: '01 01 01 02' (ST: MBB, SD: 010102)

-	SSC Mode Selection: SSC Mode 1

-	Access Type preference: 3GPP access

Rule Precedence = <lowest priority>

Traffic Descriptor: *

Route Selection Descriptor:

-	Precedence = 1

-	Network Slice Selection, S-NSSAI: '01 01 01 01' (ST: MBB, SD: 010101)

-	SSC Mode Selection: SSC Mode 1

-	DNN Selection: internet

The TT (NG-SS) transmits on the BCCH, with the following network parameters:

-	Mobile Country Code (MCC) = 246;

-	Mobile Network Code (MNC) = 081;

-	Tracking Area Code (TAC) = 000001;

-	NG-SS Cell Id = 0001 (36 bits).

The Allowed S-NSSAI list is configured in NG-SS as '01 01 01 01', '01 01 01 02' and '01 01 01 03'.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case.

##### 16.1.1.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Run initial activation. |  |  |  |
| 2 | UE > TT | Authenticate at the TT | The UE successful authenticates to PLMN 246 081. The TT indicates Allowed S-NSSAI list as: - '01 01 01 01' - '01 01 01 02' - '01 01 01 03'. | CR 1 |  |
| 3 | UE | READ EFUST and EFURSP | No URSP rules are provisioned by the PCF | CR 2 (CR 1) | A.2/1 OR A.2/2 |
| 3 | UE > TT | Initiate a connection to DNN=TestGp.rs |  |  |  |
| 4 | UE > TT | Send a PDU SESSION ESTABLISHMENT REQUEST | The UE sends the PDU SESSION ESTABLISHMENT REQUEST to the network via UL NAS TRANSPORT with: - DNN: TestGp.rs - S-NSSAI: '01 01 01 03' |  |  |
| 5 | TT > UE | Send PDU SESSION ESTABLISHMENT ACCEPT message |  | CR 3 (CR 1) |  |
| 6 | UE | UE is soft powered down |  |  |  |

| Optional step if CR 2 verification cannot be performed at step 3) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 7 | UE | READ EFUST and EFURSP |  | CR 2 | A.2/3 |

#### 16.1.1.5	Acceptance criteria

CR 1 is implicitly verified. The requirement is met if the test sequence can be executed as expected.

CR 2 is explicitly verified at step 3) by using the methods A.2/1 or A.2/2. Optional it can be verified in step 7) using method A.2/3.

CR 3 is implicitly verified if the PDU SESSION ESTABLISHMENT ACCEPT message is sent by the UE.

### 16.1.2	Support for URSP by ME

#### 16.1.2.1	Definition and applicability

If the UE has no signalled URSP and the UE has only pre-configured URSPs configured in the ME, then the UE shall use the pre-configured URSP in the ME.

The pre-configured URSP can only be used if the SUPI from the USIM matches the SUPI stored in the non-volatile memory of the ME.

#### 16.1.2.2	Conformance requirement

CR 1	The UE shall support the URSP procedure.

CR 2	The ME shall use the pre-configured URSP rules in the ME if there is no signalled URSP and UE has only pre-configured URSP in the ME.

CR 3	The URSP can only be used if the SUPI from the USIM matches the SUPI stored in the non-volatile memory of the ME.

Reference:

-	3GPP TS 24.526 [50], clause 4.2.2.2 and 5.2;

-	3GPP TS 23.503 [52], clause 6.6.2.

#### 16.1.2.3	Test purpose

The purpose of this test is to verify that the ME uses the matching URSP rule from ME to set the PDU session establishment parameters if there is no signalled URSP and there is no pre-configured URSP in the USIM.

#### 16.1.2.4	Method of test

##### 16.1.2.4.1	Initial conditions

The values of the 5G-NR UICC – support of Rel-16 features as defined in clause 4.5.11 of the present document are used with EFIMSI as defined in clause 4.6.5.

The ME is pre-configured with the following URSP rules:

USRP rules for one PLMN only

-	PLMN: 244 083

Rule Precedence = 1

Traffic descriptor:

-	DNN = TestGp.rs

Route Selection Descriptor:

-	Precedence = 1

-	Network Slice Selection, S-NSSAI: '01 01 01 02' (ST: MBB, SD: 010102)

-	SSC Mode Selection: SSC Mode 1

-	Access Type preference: 3GPP access

Rule Precedence = <lowest priority>

Traffic Descriptor: *

Route Selection Descriptor:

-	Precedence =1

-	Network Slice Selection, S-NSSAI: '01 01 01 01' (ST: MBB, SD: 010101)

-	SSC Mode Selection: SSC Mode 1

-	DNN Selection: internet

The TT (NG-SS) transmits on the BCCH, with the following network parameters:

-	Mobile Country Code (MCC) = 244;

-	Mobile Network Code (MNC) = 083;

-	Tracking Area Code (TAC) = 000001;

-	NG-SS Cell Id = 0001 (36 bits).

The Allowed S-NSSAI list is configured in NG-SS to include S-NSSAIs ('01 01 01 01', '01 01 01 01'), and ('01 01 01 02', '01 01 01 02').

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case.

##### 16.1.2.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Run initial activation. |  |  |  |
| 2 | UE > TT | Authenticate at the TT | The UE successful authenticates to PLMN 244 083. The TT indicates Allowed S-NSSAI list as: - '01 01 01 01' - '01 01 01 02'. | CR 1 |  |
| 3 | UE | READ EFUST | No URSP rules are provisioned by the PCF | CR 2 (CR 1) | A.2/1 OR A.2/2 |
| 3 | UE > TT | Initiate a connection to DNN=TestGp.rs |  |  |  |
| 4 | UE > TT | Send a PDU SESSION ESTABLISHMENT REQUEST | The UE sends the PDU SESSION ESTABLISHMENT REQUEST to the network via UL NAS TRANSPORT with: - DNN: TestGp.rs - S-NSSAI: '01 01 01 02' |  |  |
| 5 | TT > UE | Send PDU SESSION ESTABLISHMENT ACCEPT message |  | CR 3 (CR 1) |  |
| 6 | UE | UE is soft powered down |  |  |  |

| Optional step if CR 2 verification cannot be performed at step 3) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 7 | UE | READ EFUST |  | CR 2 | A.2/3 |

#### 16.1.2.5	Acceptance criteria

CR 1 is implicitly verified. The requirement is met if the test sequence can be executed as expected.

CR 2 is explicitly verified at step 3) by using the methods A.2/1 or A.2/2. Optional it can be verified in step 7) using method A.2/3.

CR 3 is implicitly verified if the PDU SESSION ESTABLISHMENT ACCEPT message is sent by the UE.

### 16.1.3	Support of Signalled URSP

#### 16.1.3.1	Definition and applicability

The HPLMN pre-configured URSP in the ME and the HPLMN signalled URSP shall be stored in a non-volatile memory in the ME together with the SUPI from the USIM.

If the UE has both pre-configured URSP(s) and signalled URSP, the UE shall only use the signalled URSP. For a UE not operating in SNPN access mode, if the UE has no signalled URSP, and the UE has pre-configured URSPs configured in both the USIM and the ME, then the UE shall use the pre-configured URSP in the USIM. The HPLMN pre-configured URSP in the ME shall be stored until a new URSP is configured by HPLMN or the USIM is removed.

#### 16.1.3.2	Conformance requirement

CR 1	The UE shall support the URSP procedure.

CR 2	If the UE has both pre-configured URSP(s) and signalled URSP, the UE shall only use the signalled URSP.

Reference:

-	3GPP TS 24.526 [50], clause 4.2.2.2 and 5.2;

-	3GPP TS 23.503 [52], clause 6.6.2.

#### 16.1.3.3	Test purpose

The purpose of this test is to verify that the ME uses signalled URSP when provided rather than pre-configured URSP in the USIM or the ME.

#### 16.1.3.4	Method of test

##### 16.1.3.4.1	Initial conditions

The values of the 5G-NR UICC – support of Rel-16 features as defined in clause 4.5.11 of the present document are used with EFIMSI as defined in clause 4.6.5 and the following exceptions:

EFUST (USIM Service Table)

Logically:

| Service n°132: |  | Support for URSP by USIM | available |
| --- | --- | --- | --- |

Coding:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary: | xxxx xx1x | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxx1 xx1x | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | … | B16 | B17 |  |  |
|  | xxxx xxxx | xxxx xxxx | xx11 xxxx | ... | xxx0 111x | 0xxx 1xxx |  |  |

EFURSP (UE Route Selection Policies)

Logically:

URSP rules for one PLMN only

-	PLMN:	246 081

Rule Precedence = 0

Traffic descriptor:

-	DNN = TestGp.rs

Route Selection Descriptor:

-	Precedence = 0

-	Network Slice Selection, S-NSSAI: '01 01 01 02' (ST: MBB, SD: 010102)

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 80 | 22 | 42 | 16 | 80 | 1E | 00 | 1C | 00 | 00 | 0C | 88 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 | B24 |
|  | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 | 00 |
|  | B25 | B26 | B27 | B28 | B29 | B30 | B31 | B32 | B33 | B34 | B35 | B36 |
|  | 0B | 00 | 09 | 00 | 00 | 06 | 02 | 04 | 01 | 01 | 01 | 02 |

The ME is pre-configured with the following URSP rules:

USRP rules for one PLMN only

-	PLMN: 246 081

Rule Precedence =1

Traffic descriptor:

-	DNN=TestGp.rs

Route Selection Descriptor:

-	Precedence=1

-	Network Slice Selection, S-NSSAI: '01 01 01 01' (ST: MBB, SD: 010101)

-	SSC Mode Selection: SSC Mode 1

-	Access Type preference: 3GPP access

The TT (NG-SS) transmits on the BCCH, with the following network parameters:

-	Mobile Country Code (MCC) = 246;

-	Mobile Network Code (MNC) = 081;

-	Tracking Area Code (TAC) = 000001;

-	NG-SS Cell Id = 0001 (36 bits).

The Allowed S-NSSAI list is configured in NG-SS as '01 01 01 01', '01 01 01 02' and '01 01 01 03'.

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case.

##### 16.1.3.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Run initial activation. |  |  |  |
| 2 | UE > TT | Authenticate at the TT | The UE successful authenticates to PLMN 246 081. The TT indicates Allowed S-NSSAI list as: - '01 01 01 01' - '01 01 01 02' - '01 01 01 03'. | CR 1 |  |
| 3 | UE | READ EFUST and EFURSP | No URSP rules are provisioned by the PCF | CR 2 (CR 1) | A.2/1 OR A.2/2 |
| 3 | UE > TT | Initiate a connection to DNN=TestGp.rs |  |  |  |
| 4 | UE > TT | Send a PDU SESSION ESTABLISHMENT REQUEST | The UE sends the PDU SESSION ESTABLISHMENT REQUEST to the network via UL NAS TRANSPORT with: - DNN: TestGp.rs - S-NSSAI: '01 01 01 02 |  |  |
| 5 | TT > UE | Send PDU SESSION ESTABLISHMENT ACCEPT message |  | (CR 1) |  |
| 6 | TT > UE | Release the PDU Session |  |  |  |
| 7 | TT > UE | Send MANAGE UE POLICY COMMAND | The UE should update the URSP rule for DNN: "TestGp.rs" Traffic Descriptor: - DNN: 'TestGp.rs' - S-NSSAI: '01 01 01 03' | CR 1 |  |
| 8 | UE > TT | Initiate a connection to DNN=TestGp.rs |  |  |  |
| 9 | UE > TT | Send a PDU SESSION ESTABLISHMENT REQUEST | The UE sends the PDU SESSION ESTABLISHMENT REQUEST to the network via UL NAS TRANSPORT with: - DNN: TestGp.rs - S-NSSAI: '01 01 01 03 |  |  |
| 10 | TT > UE | Send PDU SESSION ESTABLISHMENT ACCEPT message |  | (CR 1) |  |

| Optional step if CR 2 verification cannot be performed at step 3) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 11 | UE | READ EFUST and EFURSP |  | CR 2 | A.2/3 |

#### 16.1.3.5	Acceptance criteria

CR 1 is implicitly verified. The requirement is met if the test sequence can be executed as expected.

CR 2 is explicitly verified at step 3) by using the methods A.2/1 or A.2/2. Optional it can be verified in step 11) using method A.2/3.

# 17	CAG list handling

## 17.1	CAG list handling for 5G

### 17.1.1	Automatic CAG selection with preconfigured CAG list on USIM

#### 17.1.1.1	Definition and applicability

If MS supports CAG and is pre-configured with a non-empty "CAG information list" stored in the USIM, the MS in automatic PLMN selection mode shall consider a PLMN indicated by an NG-RAN cell only if:

- the cell is a CAG cell and broadcasts a CAG-ID for the PLMN such that there exists an entry with the PLMN ID of the PLMN in the "CAG information list" and the CAG-ID is included in the "Allowed CAG list" of the entry;

or

- the cell is not a CAG cell and:

-	there is no entry with the PLMN ID of the PLMN in the "CAG information list";

or

-	there exists an entry with the PLMN ID of the PLMN in the "CAG information list" but the "indication that the MS is only allowed to access 5GS via CAG cells" is not included in the entry.

#### 17.1.1.2	Conformance requirement

CR 1	If service n°137 is available in EFUST and a preconfigured "CAG information list" is available on the USIM, then an ME that supports CAG shall read EFCAG and consider the PLMN indicated by the NG-RAN to perform automatic PLMN selection procedure.

Reference:

-	TS 31.102 [4], clause 4.4.11.14.

CR 2	If the available CAG cell broadcasts a CAG-ID for the PLMN such that there exists an entry with the PLMN ID of the PLMN in the "CAG information list" and the CAG-ID is included in the "Allowed CAG list" of the entry, then UE shall select the CAG cell.

Reference:

-	TS 23.122 [31], clauses 3.8, 4.4.3.1.1

-	TS 24.501 [42], clauses 9.11.3.18A

#### 17.1.1.3	Test purpose

The purpose of this test is to verify that the ME only selects the CAG cell if the PLMN ID and CAG ID of the cell are found in the EFCAG (Pre-configured CAG information list) available on USIM.

#### 17.1.1.4	Method of tests

##### 17.1.1.4.1	Initial conditions

The values of the 5G-NR UICC – CAG support as defined in clause 4.5.13 of the present document are used.

The TT (NG-SS) is configured to transmit on the BCCH, with the following network parameters:

Cell A

-	TAI (MCC/MNC/TAC):		244/083/000001.

-	cag-Identity:					00 00 00 02

Cell B

-	TAI (MCC/MNC/TAC):		244/083/000002.

-	cag-Identity:					00 00 00 09

Ensure that the UE has installed and is using the UICC/USIM configuration defined for this test case and that both cells on the TT are switched off.

##### 17.1.1.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | TT | Activate NG-SS Cell B | The NG-SS Cell B on the TT is activated as defined in the initial conditions |  |  |
| 2 | UE | Run initial activation |  |  |  |
| 3 |  | Wait for 5 minutes | While waiting, the ME shall not attempt to registration to Cell B | CR 1 |  |
| 4 | TT | Activate NG-SS Cell A, deactivate NG-SS Cell B |  |  |  |
| 5 | UE > TT | Send REGISTRATION REQUEST to Cell A | The UE sends a REGISTRATION REQUEST with 5GS registration type IE as "initial registration" | CR 2 |  |
| 6 | TT > UE | Send REGISTRATION ACCEPT from Cell A | The TT sends a REGISTRATION ACCEPT with 5G-GUTI |  |  |
| 7 | UE > TT | Send REGISTRATION COMPLETE |  |  |  |
| 8 | UE | Power off/deactivate the UE |  |  |  |
| 9 | TT | Deactivate NG-SS Cell A |  |  |  |
| 10 | TT | Activate NG-SS Cell B with SIB1 modified to indicate new cag-identity 00 00 00 07 |  |  |  |
| 11 | UE | Power on/activate the UE |  |  |  |
| 12 | UE > TT | Send REGISTRATION REQUEST to Cell B | The UE sends a REGISTRATION REQUEST with 5GS registration type IE as "initial registration" | CR 2 |  |
| 13 | TT > UE | Send REGISTRATION ACCEPT from Cell B | The TT sends a REGISTRATION ACCEPT with 5G-GUTI |  |  |
| 14 | UE > TT | Send REGISTRATION COMPLETE |  |  |  |
| 15 | UE | Power off/deactivate the UE |  |  |  |
| 16 | TT | Deactivate NG-SS Cell A and Cell B |  |  |  |

#### 17.1.1.5	Acceptance criteria

CR 1 can be implicitly verified in step 3). The requirement is met if the UE is not sending a REGISTRATION REQUEST message to Cell B.

CR 2 can be implicitly verified in steps 5) and 12). The requirement is met if the UE sends REGISTRATION REQUEST message to Cell A in step 5) and to Cell B in step 6).

# 18	NSSAA procedure with SSIM

## 18.1	SSIM management procedures

### 18.1.1	SSIM identification, S-NSSAI list request

#### 18.1.1.1	Definition and applicability

If the decision of HPLMN, the SNPN or the slice owner is to require Network Slice-Specific Authentication and Authorization with a Credentials holder AAA Server (AAA-S) during the NSSAA procedure (see 3GPP TS 23.502 [61] and 3GPP TS 23.501 [39]) for a certain S-NSSAI and to require that the ME shall use the SSIM for this authentication and authorization, then the HPLMN (respectively SNPN or slice owner) shall provision a list of S-NSSAI in the SSIM that the HPLMN (respectively SNPN or slice owner) allows.

Prior to the slice establishment with AMF, the ME shall select the SSIM on the UICC to retrieve the S-NSSAI list supported by the SSIM.

#### 18.1.1.2	Conformance requirement

CR 1	If SSIM is supported on the UICC and declared in EFDIR then the ME shall read EFNSSAI from ADFSSIM to identify the S-NSSAIs supported by the SSIM.

Reference:

-	TS 31.105 [59], clauses 5.1.0. 5,1.1, 5.1.5, 7.2

CR 2	Network slice-specific authentication shall be performed if the UE indicates the support in the network slice-specific authentication and authorization in the 5GMM Capability IE in the REGISTRATION REQUEST message.

Reference:

-	TS 24.501 [25], clauses 4.6.2.4, 5.5.1.2.2 and 5.5.1.3.2

CR 3	The network slice-specific authentication and authorization procedure shall not be performed unless the primary authentication and key agreement procedure has successfully been completed.

Reference:

-	TS 24.501 [25], clauses 4.6.2.4, 5.4.1

CR 4	If one of the S-NSSAI supported by the SSIM requires authentication and authorization during NSSAA procedure with the network then the ME shall relay the EAP packets between the AAA Server (AAA-S) and the SSIM using the SSIM AUTHENTICATE command and associated NSSAA procedure NAS messages (i.e. NETWORK SLICE-SPECIFIC AUTHENTICATION COMMAND, NETWORK SLICE-SPECIFIC AUTHENTICATION COMPLETE and NETWORK SLICE-SPECIFIC AUTHENTICATION RESULT).

Reference:

-	TS 33.501 [24], clause 16.3

-	TS 23.501 [39], clauses 5.15.5.2.1, 5.15.10

-	TS 23.502 [61], clause 4.2.9

-	TS 24.501 [25], clauses 4.6.2.4, 5.4.7, 8.2.31, 8.2.32, 8.2.33

#### 18.1.1.3	Test purpose

The purpose of this test is to verify that:

- the ME identifies and selects the SSIM supported on the UICC

- the ME retrieves from the SSIM the supported S-NSSAI list from EFNSSAI

- during the NSSAA procedure requiring authentication and authorization during NSSAA procedure on a specific S-NSSAI supported by the SSIM, the ME relays the EAP packets between the AAA Server (AAA-S) and the SSIM using the SSIM AUTHENTICATE command and associated NSSAA procedure NAS messages.

#### 18.1.1.4	Method of tests

##### 18.1.1.4.1	Initial conditions

The browser's cache shall be cleared before execution of the test sequence.

The ME memory shall be cleared from all formerly stored Allowed-NSSAI and Rejected-NSSAI.

The values of the 5G-NR SSIM-UICC as defined in clause 4.5.16 of the present document are used.

The ME is pre-configured with the following Configured-NSSAI:

PLMN: 001 01

- S-NSSAI:	01 01 01 01 (SST: eMBB, SD: 010101)

- S-NSSAI:	01 01 01 02 (SST: eMBB, SD: 010102)

- S-NSSAI:	01 01 01 03 (SST: eMBB, SD: 010103)

The ME is pre-configured with the following URSP rules for one PLMN only (PLMN: 244 83):

Rule Precedence = 1

Traffic Descriptor:

- DNN = TestGp.rs

Route Selection Descriptor:

- Precedence = 1

- Network Slice Selection:	S-NSSAI: 01 01 01 03 (SST: eMBB, SD: 010103)

- SSC Mode Selection:		SSC Mode 1

- Access Type preference:		3GPP access

Rule Precedence =2

Traffic Descriptor:

- DNN = Test12.rs

Route Selection Descriptor:

- Precedence = 1

- Network Slice Selection:	S-NSSAI: 01 01 01 02 (SST: eMBB, SD: 010102)

- SSC Mode Selection:		SSC Mode 1

- Access Type preference:		3GPP access

Rule Precedence = <lowest priority>

Traffic Descriptor: *

Route Selection Descriptor:

- Precedence = 1

- SSC Mode Selection:	SSC Mode 1

- Access Type preference:	3GPP access

The TT (NG-SS) is configured to transmit on the BCCH with the following network parameters:

Cell A

-	Mobile Country Code (MCC):	244;

-	Mobile Network Code (MNC):	83;

-	Tracking Area Code (TAC):		000001;

-	NG-RAN Cell Identity value:	0001 (36 bits);

-	Subscribed S-NSSAIs:			01010102 (SST: eMBB, SD: 010102) and S-NSSAI is subject to NSSAA;

Cell B

-	Mobile Country Code (MCC):	244;

-	Mobile Network Code (MNC):	83;

-	Tracking Area Code (TAC):		000002;

-	NG-RAN Cell Identity value:	0002 (36 bits);

-	Subscribed S-NSSAIs:			01010103 (SST: eMBB, SD: 010103) and S-NSSAI is subject to NSSAA;

The ME is connected to Cell A.

Ensure that the UE has installed and is using the UICC/SSIM configuration defined for this test case.

##### 18.1.1.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Run initial activation. |  | CR 1 | A.2/1OR A.2/2 |
| 2 | UE > TT | Send REGISTRATION REQUEST | The UE sends a REGISTRATION REQUEST including Requested NSSAI: - S-NSSAI 01010101 (SST: eMBB, SD: 010101) - S-NSSAI 01010102 (SST: eMBB, SD: 010102) - S-NSSAIs 01010103 (SST: eMBB, SD: 010103) and indicating NSSAA support in 5GMM capability | CR 2 |  |
| 3 | TT > UE | Send REGISTRATION ACCEPT | The NG-SS sends a REGISTRATION ACCEPT without Allowed NSSAI and indicating "NSSAA to be performed" in 5GS registration result |  |  |
| 4 | TT < > UE | Initiate the primary authentication | The primary authentication and key agreement procedure is successfully completed | CR 3 | A.2/1 OR A.2/2 |
| 5 | ME > SSIM | Send SSIM AUTHENTICATE command | The ME relays the EAP-Request packet from EAP message IE to the SSIM using AUTHENTICATE command. |  |  |
| 6 | SSIM > ME | Send SSIM AUTHENTICATE response | The ME retrieves the EAP-Response packet from the SSIM AUTHENTICATE command response | CR 3 | A.2/1 OR A.2/2 |
| 7 | UE > TT | Send NETWORK SLICE-SPECIFIC AUTHENTICATION COMPLETE | The NETWORK SLICE-SPECIFIC AUTHENTICATION COMPLETE message includes an EAP-Response: - S-NSSAI IE set to 01010102 (SST: eMBB, SD: 010102) |  |  |
| 8 | TT > UE | Send NETWORK SLICE-SPECIFIC AUTHENTICATION RESULT | The NETWORK SLICE-SPECIFIC AUTHENTICATION RESULT message includes: - S-NSSAI IE set to 01010102 (SST: eMBB, SD: 010102) - EAP message IE containing an EAP-success or EAP-failure message | CR 4 |  |

##### 18.1.1.4.2	Acceptance criteria

CR 1 can be explicitly verified using methods A.2/1 or A.2/2 during UE initialization (step 1)). The requirement is met if the ME selects the SSIM and reads EFNSSAI.

CR 2 can be verified in step 2). The requirement is met if the UE indicates the support in the network slice-specific authentication and authorization in the 5GMM Capability IE in the REGISTRATION REQUEST message

CR 3 can explicitly be verified in steps 4) and 6) using methods A.2/1 or A.2/2.

CR 4 can be verified in step 8). It is met if the NETWORK SLICE-SPECIFIC AUTHENTICATION RESULT message includes:

- -	S NSSAI IE set to 01010102 (SST: eMBB, SD: 010102);

- -	EAP message IE containing an EAP-success or EAP-failure message.

## 18.2	SSIM commands

### 18.2.1	AUTHENTICATE command, Identity EAP-Request/EAP-Response messages

#### 18.2.1.1	Definition and applicability

If the decision of HPLMN, the SNPN or the slice owner is to require Network Slice-Specific Authentication and Authorization with a Credentials holder AAA Server (AAA-S) during the NSSAA procedure (see 3GPP TS 23.502 [61] and 3GPP TS 23.501 [39]) for a certain S-NSSAI and to require that the ME shall use SSIM for this authentication and authorization, then the HPLMN (respectively SNPN or slice owner) shall provision associated identifier (i.e. EAP Identifier).

At the beginning of the Network Slice-Specific Authentication and Authorization with a Credentials holder AAA Server (AAA-S) on supported S-NSSAI for a given SSIM, the ME shall:

- relay the EAP-Request/Identity packet from EAP message IE in the first NETWORK SLICE-SPECIFIC AUTHENTICATION COMMAND NAS message to the SSIM using AUTHENTICATE command.

- relay the EAP-Response/Identity packet returned by the SSIM on AUTHENTICATE command to EAP message IE in the first NETWORK SLICE-SPECIFIC AUTHENTICATION COMPLETE NAS message.

#### 18.2.1.2	Conformance requirement

CR 1	If one of the S-NSSAI supported by the SSIM requires authentication and authorization during NSSAA procedure with the network, then the ME shall relay the EAP-Request/Identity packet included in the first NETWORK SLICE-SPECIFIC AUTHENTICATION COMMAND NAS message from the network to the appropriate SSIM using the SSIM AUTHENTICATE command.

CR 2	If one of the S-NSSAI supported by the SSIM requires authentication and authorization during NSSAA procedure with the network, then the ME shall relay the EAP-Response/Identity packet included in the response of the first SSIM AUTHENTICATE command sent to the appropriate SSIM to the first NETWORK SLICE-SPECIFIC AUTHENTICATION COMPLETE NAS message sent to the network.

Reference:

-	TS 31.105 [59], clauses 4.2.2, 7.2

-	TS 33.501 [24], clause 16.3

-	TS 23.501 [39], clauses 5.15.5.2.1, 5.15.10

-	TS 23.502 [61], clause 4.2.9

-	TS 24.501 [25], clauses 5.4.7, 8.2.31, 8.2.32, 8.2.33

#### 18.2.1.3	Test purpose

The purpose of this test is to verify that at the beginning of the NSSAA procedure at the beginning of the NSSAA procedure requiring authentication and authorization during NSSAA procedure on a specific S-NSSAI supported by the SSIM,:

- the ME relays the EAP-Request/Identity packet from EAP message IE in the first NETWORK SLICE-SPECIFIC AUTHENTICATION COMMAND NAS message to the SSIM using AUTHENTICATE command.

- the ME relays the EAP-Response/Identity packet returned by the SSIM on AUTHENTICATE command to EAP message IE in the first NETWORK SLICE-SPECIFIC AUTHENTICATION COMPLETE NAS message.

#### 18.2.1.4	Method of tests

##### 18.2.1.4.1	Initial conditions

The initial conditions defined in 18.1.1.4.1 apply.

##### 18.2.1.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Run initial activation. |  |  |  |
| 2 | UE > TT | Send REGISTRATION REQUEST | The UE sends a REGISTRATION REQUEST including Requested NSSAI: - S-NSSAI 01010101 (SST: eMBB, SD: 010101) - S-NSSAI 01010102 (SST: eMBB, SD: 010102) - S-NSSAIs 01010103 (SST: eMBB, SD: 010103) and indicating NSSAA support in 5GMM capability |  |  |
| 3 | TT > UE | Send REGISTRATION ACCEPT | The NG-SS sends a REGISTRATION ACCEPT without Allowed NSSAI and indicating "NSSAA to be performed" in 5GS registration result |  |  |
| 4 | TT < > UE | Initiate the primary authentication | The primary authentication and key agreement procedure is successfully completed |  |  |
| 5 | ME > SSIM | Send SSIM AUTHENTICATE command | The ME relays the EAP-Request packet from EAP message IE to the SSIM using AUTHENTICATE command. | CR 1 | A.2/1 OR A.2/2 |
| 6 | SSIM > ME | Send SSIM AUTHENTICATE response | The ME retrieves the EAP-Response packet from the SSIM AUTHENTICATE command response |  |  |
| 7 | UE > TT | Send NETWORK SLICE-SPECIFIC AUTHENTICATION COMPLETE | The NETWORK SLICE-SPECIFIC AUTHENTICATION COMPLETE message includes an EAP-Response: - S-NSSAI IE set to 01010102 (SST: eMBB, SD: 010102) | CR 2 |  |
| 8 | TT > UE | Send NETWORK SLICE-SPECIFIC AUTHENTICATION RESULT | The NETWORK SLICE-SPECIFIC AUTHENTICATION RESULT message includes: - S-NSSAI IE set to 01010102 (SST: eMBB, SD: 010102) - EAP message IE containing an EAP-success or EAP-failure message |  |  |

##### 18.2.1.4.3	Acceptance criteria

CR 1 can be verified in step 5) using methods A.2/1 or A.2/2. The requirement is met if the ME relays the EAP-Request packet from EAP message IE to the SSIM using AUTHENTICATE command.

CR 2 can be verified in step 7). The requirement is met if:

- the returned EAP-Response/Identity is verylongusername1@3gpp.com

- all AUTHENTICATE commands are ODD INS only with a BER-TLV structure,

- only for 'First block of authentication data' (b8b7b6 of P1 set to '100') AUTHENTICATE command the S-NSSAI field is present in command data and set to 01010102.

### 18.2.2	AUTHENTICATE command, other EAP-Request/EAP-Response messages

#### 18.2.2.1	Definition and applicability

If the decision of HPLMN, the SNPN or the slice owner is to require Network Slice-Specific Authentication and Authorization with a Credentials holder AAA Server (AAA-S) during the NSSAA procedure (see 3GPP TS 23.502 [61] and 3GPP TS 23.501 [39]) for a certain S-NSSAI and to require that the ME shall use SSIM for this authentication and authorization, then the HPLMN (respectively SNPN or slice owner) shall provision associated identifier (i.e. EAP Identifier) and credentials in the SSIM that the HPLMN (respectively SNPN or slice owner) allows.

During the Network Slice-Specific Authentication and Authorization with a Credentials holder AAA Server (AAA-S) on supported S-NSSAI for a given SSIM, the ME shall:

- relay the EAP-Request packet from EAP message IE in the NETWORK SLICE-SPECIFIC AUTHENTICATION COMMAND NAS message to the SSIM using AUTHENTICATE command.

- relay the EAP-Response packet returned by the SSIM on AUTHENTICATE command to EAP message IE in the NETWORK SLICE-SPECIFIC AUTHENTICATION COMPLETE NAS message.

#### 18.2.2.2	Conformance requirement

CR 1	If one of the S-NSSAI supported by the SSIM requires authentication and authorization during NSSAA procedure with the network, then the ME shall relay the EAP-Request packet included in the NETWORK SLICE-SPECIFIC AUTHENTICATION COMMAND NAS message from the network to the appropriate SSIM using the SSIM AUTHENTICATE command.

CR 2	If one of the S-NSSAI supported by the SSIM requires authentication and authorization during NSSAA procedure with the network, then the ME shall relay the EAP-Response packet included in the response of the  SSIM AUTHENTICATE command from the SSIM to the NETWORK SLICE-SPECIFIC AUTHENTICATION COMPLETE NAS message sent to the network.

Reference:

-	TS 31.105 [59], clauses 7.2

-	TS 33.501 [24], clause 16.3

-	TS 23.501 [39], clauses 5.15.5.2.1, 5.15.10

-	TS 23.502 [61], clause 4.2.9

-	TS 24.501 [25], clauses 5.4.7, 8.2.31, 8.2.32, 8.2.33

#### 18.2.2.3	Test purpose

The purpose of this test is to verify that during the NSSAA procedure requiring authentication and authorization on a specific S-NSSAI supported by the SSIM, the ME relays:

- the EAP-Request packet from EAP message IE in the NETWORK SLICE-SPECIFIC AUTHENTICATION COMMAND NAS message to the SSIM using AUTHENTICATE command.

- the EAP-Response packet returned by the SSIM on AUTHENTICATE command to EAP message IE in the NETWORK SLICE-SPECIFIC AUTHENTICATION COMPLETE NAS message.

#### 18.2.2.4	Method of tests

##### 18.2.2.4.1	Initial conditions

The initial conditions defined in 18.2.1.4.1 apply.

##### 18.2.2.4.2	Procedure

The procedure defined in 18.2.1.4.2 is used.

##### 18.2.2.4.3	Acceptance criteria

CR 1 can be verified in step 5) using methods A.2/1 or A.2/2. The requirement is met if the ME relays the EAP-Request packet from EAP message IE to the SSIM using AUTHENTICATE command.

CR 2 can be verified in step 7). The requirement is met if:

- the returned EAP-Response from the AUTHENTICATE command response to EAP message IE is included in the NETWORK SLICE-SPECIFIC AUTHENTICATION COMPLETE message,

- all AUTHENTICATE commands are ODD INS only with a BER-TLV structure

- only for 'First block of authentication data' (b8b7b6 of P1 set to '100') AUTHENTICATE command the S-NSSAI field is present in command data and set to 01010102

- the EAP message does not fit in one single SSIM AUTHENTICATE command, following data of the EAP message are sent segmented using 'Next block of authentication data' (b8b7b6 of P1 set to '000') AUTHENTICATE command and the S-NSSAI field is not present in command data.

### 18.2.3	AUTHENTICATE command, EAP-Success/EAP-Failure messages

#### 18.2.3.1	Definition and applicability

At the end of the Network Slice-Specific Authentication and Authorization with a Credentials holder AAA Server (AAA-S) on supported S-NSSAI for a given SSIM, the ME shall:

- relay the EAP-success/EAP-failure packet from EAP message IE in the NETWORK SLICE-SPECIFIC AUTHENTICATION RESULT NAS message to the SSIM using AUTHENTICATE command.

#### 18.2.3.2	Conformance requirement

CR 1	If one of the S-NSSAI supported by the SSIM requires authentication and authorization during NSSAA procedure with the network, then the ME shall relay the EAP-success/EAP-failure packet included in EAP message IE from the NETWORK SLICE-SPECIFIC AUTHENTICATION RESULT NAS message issued by the network to the appropriate SSIM using the SSIM AUTHENTICATE command.

Reference:

-	TS 31.105 [59], clauses 7.2

-	TS 33.501 [24], clause 16.3

-	TS 23.501 [39], clauses 5.15.5.2.1, 5.15.10

-	TS 23.502 [61], clause 4.2.9

-	TS 24.501 [25], clauses 5.4.7, 8.2.31, 8.2.32, 8.2.33

#### 18.2.3.3	Test purpose

The purpose of this test is to verify that, at the end of the NSSAA procedure requiring authentication and authorization on a specific S-NSSAI supported by the SSIM, the ME relays EAP-success/EAP-failure packet from EAP message IE in the NETWORK SLICE-SPECIFIC AUTHENTICATION RESULT NAS message to the SSIM using AUTHENTICATE command.

#### 18.2.3.4	Method of tests

##### 18.2.3.4.1	Initial conditions

The initial conditions defined in 18.2.1.4.1 apply.

##### 18.2.3.4.2	Procedure

| Step | Direction | Action | Information | REQ | SA |
| --- | --- | --- | --- | --- | --- |
| 1 | UE | Run initial activation. |  |  |  |
| 2 | UE > TT | Send REGISTRATION REQUEST | The UE sends a REGISTRATION REQUEST including Requested NSSAI: - S-NSSAI 01010101 (SST: eMBB, SD: 010101) - S-NSSAI 01010102 (SST: eMBB, SD: 010102) - S-NSSAIs 01010103 (SST: eMBB, SD: 010103) and indicating NSSAA support in 5GMM capability |  |  |
| 3 | TT > UE | Send REGISTRATION ACCEPT | The NG-SS sends a REGISTRATION ACCEPT without Allowed NSSAI and indicating "NSSAA to be performed" in 5GS registration result |  |  |
| 4 | TT < > UE | Initiate the primary authentication | The primary authentication and key agreement procedure is successfully completed |  |  |
| 5 | ME > SSIM | Send SSIM AUTHENTICATE command | The ME relays the EAP-Request packet from EAP message IE to the SSIM using AUTHENTICATE command. |  |  |
| 6 | SSIM > ME | Send SSIM AUTHENTICATE response | The ME retrieves the EAP-Response packet from the SSIM AUTHENTICATE command response |  |  |
| 7 | UE > TT | Send NETWORK SLICE-SPECIFIC AUTHENTICATION COMPLETE | The NETWORK SLICE-SPECIFIC AUTHENTICATION COMPLETE message includes an EAP-Response: - S-NSSAI IE set to 01010102 (SST: eMBB, SD: 010102) |  |  |
| 8 | TT > UE | Send NETWORK SLICE-SPECIFIC AUTHENTICATION RESULT | The NETWORK SLICE-SPECIFIC AUTHENTICATION RESULT message includes: - S-NSSAI IE set to 01010102 (SST: eMBB, SD: 010102) - EAP message IE containing an EAP-success or EAP-failure message | CR 1 |  |

##### 18.2.3.4.3	Acceptance criteria

CR 1 can be verified in step 8). The requirement is met if:

- the ME relays the EAP-Request packet from EAP message IE to the SSIM using AUTHENTICATE command,

- the ME relays to the appropriate SSIM supporting the S-NSSAI 01010102 the EAP-success or EAP-failure from EAP message IE received on NETWORK SLICE-SPECIFIC AUTHENTICATION RESULT message using AUTHENTICATE command,

- the AUTHENTICATE command is ODD INS only with a BER-TLV structure.

- only for 'First block of authentication data' (b8b7b6 of P1 set to '100') AUTHENTICATE command the S-NSSAI field is present in command data and set to 01010102.

Annex A (informative):
Suggested requirements lists for Test Applet functionality

A Test Applet should be installed on to a nrUICC for testing USAT functionalities or testing USIM functionalities (e.g. test enviornment in clause 4.1.5) if toolkit applet is required. Test Applet and TT should support general requirements in clause A.1 and the requirements in either the List-1 or List-2 in clause A.2 for test execution.

# A.1	General requirements

|  | General requirements |
| --- | --- |
| A.1.1 | Test instructions or Test Input data for test case execution should be read from Test EFs. |
| A.1.2 | Format of the test instructions input data should be defined as per the applet implementation. |
| A.1.3 | Applet should be able to trigger proactive commands using test case instructions or input data Test EFs. |
| A.1.4 | Applet should be able to handle Call Control events from ME and return treatments using test case instructions or input data Test EFs. |
| A.1.5 | TT should be able to instruct the applet when to read individual test instructions for executing them sequentially and should maintain proper synchronization with the test steps executed on the TT (Network Simulator). |
| A.1.6 | Applet should be capable of resuming test steps in a test case after a device reset or UICC RESET refresh if required (e.g. Device reset or the REREFRESH is part of the test case). |
| A.1.7 | Applet should be able to register or deregister for toolkit events in the ME - nrUICC interface. NOTE 1: A SETUP EVENT LIST proactive command will be triggered by the nrUICC due to the event registration or deregistration. NOTE 2: Applet may not receive the TERMINAL RESPONSE (TR) for the SETUP EVENT LIST proactive command. TT may use other methods to verify the TR content if required. |
| A.1.8 | Applet should be capable of processing concatenated envelopes (e.g., in the case of Long SoR) and to take the necessary actions according to the test requirements. |
| A.1.9 | Applet should be able to read EF contents from a specific test file (e.g. EFEF_RESET) to restore data in dedicated / standard EFs to its values in the original TS.48 configuration (e.g. prior to executing a test case). NOTE 3: Applet may trigger UICC RESET after restoring the EF data. |
| A.1.10 | Loading Test toolkit applets onto the nrUICC should be possible by using widely available tools. |
| A.1.11 | Test applet should support events defined in ETSI TS 102 241 Rel-18 and 3GPP 31.130, |

# A.2	Test instruction or input data requirements

Test applet should be able to read test instruction or input data from EFs under DFTEST or from EFTC_IN. as defined in requirements list 1 or list 2 below.

## A.2.1	Test instruction or input data requirements list-1

Requirements in the following list are suggested to be supported within the test environment for seamless testing as defined in clause 4.1.4 of the present document.

|  | Requirements for Seamless testing environment |
| --- | --- |
| A.2.1.1 | TT or User should be able to configure the necessary Elementary Files (EFSTK, EFSETSTK, EFCC, EFEVENTLIST) of the activated USIM/ISIM application using test specific configuration data prior to execution of the test procedure. |

## A.2.2	Test instruction or input data requirements list-2

Requirements in the following list are suggested to be supported within the test environment for Test toolkit events-based testing as defined in clause 4.1.5 of the present document.

|  | Requirements for Test events-based testing |
| --- | --- |
| A.2.2.1 | Test instructions or Input data for test case execution should be read from the test EF (EFTC_IN) configurable by the TT prior to execution of the test procedure (during test case initialization). |

# A.3	APDU content verification requirements

## A.3.1	APDU content verification requirements list-1

Requirements in the following list are suggested to be supported within the test environment for seamless testing as defined in clause 4.1.4 of the present document.

|  | Requirements for Seamless testing environment |
| --- | --- |
| A.3.1.1 | TT should be able to receive and process the APDU TX/RX data exchanged between UICC and ME while maintaining proper synchronization with the test steps executed in the TT. |
| A.3.1.2 | Verification of APDU TX/RX data for the respective EF should be possible by the TT during or after the test case execution. |

## A.3.2	APDU content verification requirements list-2

Requirements in the following list are suggested to be supported within the test environment for seamless testing as defined in clause 4.1.5 of the present document.

|  | Requirements for Test events-based testing |
| --- | --- |
| A.3.2.1 | Applet should be capable of writing the contents of selected APDUs (e.g.: TERMINAL RESPONSE, ENVELOPE data, any selected APDUs etc), received from the ME - nrUICC interface to a test output file (e.g. EFTC_OUT). |
| A.3.2.2 | TT should read EFTC_OUT file after the test case exedution to verify the acceptance criteria. |
| A.3.2.3 | Format of the verification data in the test output file should be defined as per the applet implementation. |
| A.3.2.4 | Applet should be able to process the TERMINAL RESPONSE received from the ME upon triggering a proactive command from the Applet. |
| A.3.2.5 | When required by a test case, applet should be able to register or deregister for receiving an ADPU to verify its content. NOTE 1: Test events based APDU verification (clause 4.1.5) will require additional test applet support for the events EVENT_TEST_EXTERNAL_FILE_READ and EVENT_TEST_RX_APDU defined in ETSI TS 102 241 Rel 18. |

Annex B (informative):
Examples of Test EFs in the Test-nrUICC configuration

The details provided for the Test-SIM (Default UICC) in TS 31.121 [2] clause 4 apply to the Test-nrUICC configuration used with this test specification, with the following addition.

Test Tool or device manufacturer may provide the Test EF structure on the GSMA TS.48 [9] configuration depending on the test applet used for the testing.

# B.1	Test EF structure - 1

DFTEST shall be present at the Master File (MF) level to be used for testing purpose.

The general structure of DFTEST and associated EF is defined below.

This EF structure is suggested to be used with the test environment defined in clause 4.1.4 of the present document in case USAT functionality is required to operate the test case.

|  |  |  |  |  |  |  |  |  |  |  | MF |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  | '3F00' |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | EFDIR |  |  | EFPL |  |  | EFARR |  |  | EFICCID |  |  | EFUMPC |  |  |
|  |  |  |  |  |  |  |  | '2F00' |  |  | '2F05' |  |  | '2F06' |  |  | '2FE2' |  |  | '2F08' |  |  |
|  |  | DFTEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | '7F0A' |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | EFSTK |  |  | EFSETSTK |  |  | EFCC |  |  | EFEVENTLIST |  |  |  |  |  |
|  |  |  |  |  |  |  |  | '2F0A' |  |  | '2F0B' |  |  | '2F0C' |  |  | '2F0D' |  |  |  |  |  |

The information in EFSTK, EFSETSTK, EFCC, EFEVENTLIST shall be used. A Generic STK Applet that utilizes the test data configured by tester in these new EFs to trigger the required commands from the UICC shall be used.

NOTE:	File ID for DFTEST is implementation specific.

## B.1.1	EFSTK (SIMToolkit data)

| Identifier: '2F0A' |  | Structure: linear fixed |  |  | Optional |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Record length: X bytes (see note) |  |  |  | Update activity: low |  |  |  |
| Access Conditions: READ ALW UPDATE ALW DEACTIVATE ALW ACTIVATE ALW |  |  |  |  |  |  |  |
| Bytes | Description |  | Default Value |  |  | M/O | Length |
| 1 to X | Test Data - Record 1 |  | 'FF FF .. FF FF' |  |  | M | X bytes |
| .. | .. |  | .. |  |  | M | X bytes |
| 39*X+1 to 40*X | Test Data - Record 40 |  | 'FF FF .. FF FF' |  |  | M | Xbytes |

Each individual record can hold the tester configured data, specific to the proactive command that needs to be issued by the ME referenced by EFSETSTK, for structure and coding details of the proactive command refer TS 31.111 [26].

NOTE:	The record length is implementation specific. It is recommended to configure a record length allowing to entirely store each of the toolkit commands required for testing.

## B.1.2	EFSETSTK (Set SIM Toolkit)

| Identifier: '2F0B' |  | Structure: transparent |  |  | Optional |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| File size: 1 byte |  |  |  | Update activity: low |  |  |  |
| Access Conditions: READ ALW UPDATE ALW DEACTIVATE ALW ACTIVATE ALW |  |  |  |  |  |  |  |
| Bytes | Description |  | Default Value |  |  | M/O | Length |
| 1 | Test Data |  | '00' |  |  | M | 1 byte |

Test Data is either set by the tester to the default value '0x00' or a specific record number of EFSTK. When read whilst set to a specific record number of EFSTK, it will imply to trigger the respective SIM toolkit command.

Example:

EFSTK is configured as '0x01':

-	the test toolkit applet will issue the command stored at record 01 of EFSTK.

-	Once the command is issued successfully, the applet will set EFSETSTK to the default value.

EFSTK is configured as '0x80':

-	the test toolkit applet will issue all available commands stored in records of EFSTK in sequence.

NOTE:	This is helpful for tests where multiple proactive commands are required to be issued in sequence (from record 1 to n of EFSTK) after receiving the Terminal Response from the prior command.

## B.1.3	EFCC (CALL CONTROL)

| Identifier: '2F0C' |  | Structure: transparent |  |  | Optional |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Record length: 255 bytes |  |  |  | Update activity: low |  |  |  |
| Access Conditions: READ ALW UPDATE ALW DEACTIVATE ALW ACTIVATE ALW |  |  |  |  |  |  |  |
| Bytes | Description |  | Default Value |  |  | M/O | Length |
| 1 to 255 | Test Data |  | '00 .. 00' |  |  | M | 255 bytes |

Test Data is configured by the tester with the Call control response (as defined in TS 31.111 [26]) to process the Call Control Envelope commands received from the ME based on the Call Control service configured in UST.

For example:

1.	'00 00' implies allowed not modified.

2.	'01 00' implies not allowed.

3.	'02 LL…XX' implies allowed with modifications. (LL denotes the length of the TLV).

## B.1.4	EFEVENTLIST (EVENT LIST)

| Identifier: '2F0D' |  | Structure: transparent |  |  | Optional |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Record length: 255 bytes |  |  |  | Update activity: low |  |  |  |
| Access Conditions: READ ALW UPDATE ALW DEACTIVATE ALW ACTIVATE ALW |  |  |  |  |  |  |  |
| Bytes | Description |  | Default Value |  |  | M/O | Length |
| 1 to 255 | Test Data |  | 'FF .. FF' |  |  | M | 255 bytes |

The Test data is of format <length><event1><event2>..<event n>.. FF FF, where the length indicates the number of configured events as configured by the tester. Supported events are defined in TS 31.111 [26].

The data persist even after the card reset. Hence this data needs to be cleared by the tester by reverting the contents to default values (FF..FF)

# B.2	Test EF structure - 2

EFTC_IN, EFTC_OUT, and EFEF_RESET files may be used for testing purpose and if used, shall be present at the Master File (MF) level. A Generic STK Test Applet can make use of the data configured in those files by the TT.

The general structure for the EFs is defined as below.

|  |  |  |  |  |  | MF |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | '3F00' |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | EFDIR |  |  | EFPL |  |  | EFARR |  |  | EFICCID |  |  |  | EFUMPC |  |
|  |  |  | '2F00' |  |  | '2F05' |  |  | '2F06' |  |  | '2FE2' |  |  |  | '2F08' |  |
|  |  |  | EFEF_RESET |  |  | EFTC_IN |  |  | EFTC_OUT |  |
|  |  |  | '2FXX' |  |  | '2FYY' |  |  | '2FZZ' |  |

NOTE:	File IDs for EFTC_IN, EFTC_OUT, and EFEF_RESET files are implementation specific.

This EF structure is suggested to be used with the test environment defined in clause 4.1.5 of the present document in case USAT functionality is required to operate the test case.

EFTC_IN and EFTC_OUT files are used during test case execution, and it shall be updated fresh for each test case. EFEF_RESET file is used for saving EF content required to restore the TS.48 configuration to its original content before initializing test specific data.

## B.2.1	EFTC_IN (Test Case Input)

This file contains data required for a test applet to handle test case execution. Content and format of the data may be changed depending on the Applet implementation.

| Identifier: '2FXX' |  | Structure: transparent |  |  | Optional |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Record length: 255 bytes |  |  |  | Update activity: low |  |  |  |
| Access Conditions: READ ALW UPDATE ALW DEACTIVATE ALW ACTIVATE ALW |  |  |  |  |  |  |  |
| Bytes | Description |  | Default Value |  |  | M/O | Length |
| 1 to 255 | Test Data |  | 'FF .. FF' |  |  | M | 255 bytes |

The EF may include data related to:

Test step control

-	TT can update this data to direct test applet to move to next step in the test case.

Toolkit command data

-	This can be a sequence of TLVs related to proactive commands or event registrations or call control response data or any other action required for the test case.

Different tags can be defined for this purpose.

Example:

| Bytes | Description | M/O | Length |
| --- | --- | --- | --- |
| 1 | Test step control | M | 1 byte |
| 2 | Tag 1: Wait for new step tag | M | 1 byte |
| 3 | Tag 1 length | M | 1 byte |
| 4 to 4+X-1 | Tag 1 data | M | X bytes |
| 4+X | Tag 2: Proactive command tag | O | 1 byte |
| 5+X | Tag 2 length | O | 1 byte |
| 6+X to 6+X+Y-1 | Tag 2 data | O | Y bytes |
| NOTE: X and Y is greater than zero if the Tag is present. |  |  |  |

## B.2.2	EFTC_OUT (Test Case Output)

This file contains data received from the ME and it can be read by the TT to verify if meeting the acceptance criteria of a test case. Additionally, few leading bytes can be used for tracking EFTC_IN and EFTC_OUT current offsets if required by the applet. Content and format of the data may be changed depending on the Applet implementation.

| Identifier: '2FYY' |  | Structure: transparent |  |  | Optional |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Record length: 255 bytes |  |  |  | Update activity: low |  |  |  |
| Access Conditions: READ ALW UPDATE ALW DEACTIVATE ALW ACTIVATE ALW |  |  |  |  |  |  |  |
| Bytes | Description |  | Default Value |  |  | M/O | Length |
| 1 to 255 | Test Data |  | 'FF .. FF' |  |  | M | 255 bytes |

The EF can include data related to,

Test EFs offset control,

-	TT can update initial offsets during test case initialization.

-	TT can track and update the test EF offsets as applet executes test steps.

Toolkit command response data received from the ME,

-	This can be a sequence of TLVs related to proactive command responses (TERMINAL RESPONSE) or envelope commands (EVENTs).

-	Different Tags can be used for this purpose.

Example:

| Bytes | Description | M/O | Length |
| --- | --- | --- | --- |
| 1-2 | Test EFs offset control bytes | M | 2 bytes |
| 3-4 | Length | M | 2 bytes |
| 5 | Tag 1: Proactive command 1 tag (for TERMINAL RESPONSE 1) | O | 1 byte |
| 6 | Tag 1 length | O | 1 byte |
| 7 to 7+X-1 | Tag 1 data | O | X bytes |
| 7+X | Tag 2: Proactive command 2 tag (for TERMINAL RESPONSE 2) | O | 1 byte |
| 8+X | Tag 2 length | O | 1 byte |
| 9+X to 9+X+Y-1 | Tag 2 data | O | Y bytes |
| NOTE: X and Y is greater than zero if the Tag is present. |  |  |  |

## B.2.3	EFEF_RESET (EF_RESET)

This file contains data required for updating EFs in the TS.48 configuration in order to restore the original content of it as part of test case initialization. TT can request the applet through test control byte to read the content from this EF and update required EFs in the TS.48 configuration. Content and format of the data may be changed depending on the Applet implementation.

[…]

| Identifier: '2FZZ' |  | Structure: linear fixed |  |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Record length: 128 bytes |  |  | Update activity: low |  |  |  |  |
| File size: 128 * n, (n = 25) |  |  |  |  |  |  |  |
| Access Conditions: READ ALW UPDATE ALW DEACTIVATE ALW ACTIVATE ALW |  |  |  |  |  |  |  |
| Byte | Description |  |  | Default Value | M/O |  | Length |
| 1 to X(1) | EF content read control data |  |  | 'FF FF .. FF FF' | M |  | X bytes |
| X+1 to X+1+128-1 | Sequence of EF content data - 1 |  |  | 'FF FF .. FF FF' | M |  | 128 bytes |
| X+1+128 to X+1+128*2-1 | Sequence of EF content data - 2 |  |  | 'FF FF .. FF FF' | O |  | 128 bytes |
| .. | .. |  |  | .. | O |  | 128 bytes |
| X+1+128*23 to X+1+128*24-1 | Sequence of EF content data - 24 |  |  | 'FF FF .. FF FF' | O |  | 128 bytes |
| NOTE 1: X depends on applet implementation and is greater than zero. |  |  |  |  |  |  |  |

The EF can include data related to,

EF content read control data,

-	TT can update this record with an instruction on what records to be read to update the EFs in the nrUICC.

Sequence of EF content data,

-	This can be a sequence of TLVs related to EF content in the TS.48 configuration.

-	Each sequence can include data related to more than one EF in the TS.48 configuration.

-	Applet can choose a format to include number of padding pattern (eg ‘FF’) or unused bytes instead of updating with a sequence of same pattern to save the size of data in a record.

Example format:

| Record number | Data format |
| --- | --- |
| 1 | <instruction to read all none empty records>, <REFRESH flag> OR <read from start record>, <read end record>, <REFRESH flag> |
| 2 | <path length 1>,< EF id 1>,<0 (for Transparent EF)>,<length of data 1>,<data 1>,<no of repeated pattern>,<length of pattern>,<repeated pattern>, <path length 2>,< EF id 2>,<1, (for record# 1)>,<length of actual data 2.1>,<data 2.1>,<no of repeated pattern>, <length of pattern>,<repeated pattern>, <path length 2>,< EF id 2>,<2, (for record# 2)>,<length of actual data 2.2>,<data 2.2>,<no of repeated pattern>, <length of pattern>,<repeated pattern>, ….. |
| 3 | <path length 5>,< EF id 5>,<0 (Transparent EF)>,<length of data 5>,<data 5>,0, <path length 6>,< EF id 6>,<1 (Record# 1)>,<no. of FF>,<length of actual data 6.1>,<data 6.1>,0, <path length 7>,< EF id 7>,<0 (Transparent E)>,<no. of FF>,<length of actual data 7>,<data 7>,0, ….. |
| NOTE: <length of pattern> and <repeated pattern> is not required if <no of repeated pattern> is 0. |  |

Example data:

| Record number | Data format |
| --- | --- |
| 1 | FD,1 OR 2 ,3, 0 |
| 2 | '02 6F C9 01 04 01 02 03 04 00…..' |
| 3 | '02 6F CE 05 01 09 07 01 FF 02 6F D2 00 03 03 03 03 00 61 01 FF…..' |

Annex C (informative):
Add a title

# C.1	5G-NR generic procedure for 5G Registration on 3GPP access with IMS service

## C.1.1	'voice centric' Procedure

| Step | Direction | Action | Information |
| --- | --- | --- | --- |
| 1 | TT | NG-SS Cell is powered up with SIB1 as defined in the test case initial conditions and the additional requirements as defined in clause 4.7. | pc_VOICE_CENTRIC is TRUE. The NG-SS on the TT is activated. |
| 2 | UE | Run initial activation |  |
| 3 | UE <> TT | UE camps on NG-SS Cell and establishes RRC connection |  |
| 4 | UE > TT | Send REGISTRATION REQUEST |  |
| 5 | TT > UE | Send AUTHENTICATION REQUEST |  |
| 6 | UE | UE passes the RAND and AUTN values to the USIM |  |
| 7 | UE > TT | Send AUTHENTICATION RESPONSE | The RES value in AUTHENTICATION RESPONSE message is calculated according to TS 24.501 [42]. It is equal to the XRES calculated at the TT |
| 8 | TT > UE | Send SECURITY MODE COMMAND |  |
| 9 | UE > TT | Send SECURITY MODE COMPLETE |  |
| 10 | TT > UE | Send REGISTRATION ACCEPT |  |
| 11 | UE > TT | Send REGISTRATION COMPLETE |  |
| 12 | UE > TT | Send an ULInformationTransfer message with a PDU SESSION ESTABLISHMENT REQUEST for the DNN type 'ims'. | 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT REQUEST (DNN=IMS) |
| 13 | TT > UE | Send a RRCReconfiguration message with an PDU SESSION ESTABLISHMENT ACCEPT | 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT ACCEPT |
| 14 | UE > TT | Send RRCReconfigurationComplete |  |
| 15 | UE <> TT | Run IMS Registration | The IMS Registration procedure is executed in accordance with TS 34.229-5 [58] clause A.2 (steps 1-8). |

Annex D (informative):
APDU logging in seamless testing

If the device manufacturer provides the option to facilitate the APDU logging in seamless testing (see Table A.2), the monitoring of data transfer seamlessly between ME and nrUSIM can be transferred to the TT to determine if conformance requirements are met.

# D.1	Example of SIM APDU logging via AT interface

In this example, the AT command (enable logging) starts the APDU logging, and AT command (disable logging) stops the APDU logging as defined in 3GPP TS 27.007 [16]). The APDU data is made available to TT directly or indirectly via the AT interface.

Figure D.1: Example of APDU logging via AT interface

Annex E (informative):
Change history

| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2023-08 | CT6#116 | C6-230499 |  |  |  | Initial version for information |  |
| 2023-11 | CT6#117 | C6-230664 |  |  |  | Release candidate | 1.1.0 |
| 2023-11 | CT6#117 | C6-230738 |  |  |  | Version in CT6 meeting | 2.0.0 |
| 2023-12 | CT#102 | CP-233094 |  |  |  | Presented for information and approval | 2.0.1 |
| 2023-12 | CT#102 |  |  |  |  | TS approved in TSG#102 | 17.0.0 |
| 2024-03 | CT#103 | CP-240135 | 0001 | - | F | Correction of codings in the 5G test UICC definitions | 17.1.0 |
| 2024-03 | CT#103 | CP-240135 | 0002 | - | F | Addition of test case | 17.1.0 |
| 2024-03 | CT#103 | CP-240135 | 0003 | - | F | Insertion of a clause 17 header and correction of TC 17.1.1 | 17.1.0 |
| 2024-06 | CT#104 | CP-241209 | 0004 | 1 | F | Correction of PIN handling tests | 17.2.0 |
| 2024-12 | CT#106 | CP-243162 | 0007 | 2 | F | TS 31.127 Annex Z Test toolkit applet requirements | 17.3.0 |
| 2024-12 | CT#106 | CP-243163 | 0008 | 2 | F | TS 31.127 Correction to Test Env in section 4 and Annex A | 18.0.0 |
| 2025-03 | CT#107 | CP-250065 | 0009 | - | F | Correction in default value definitions | 18.1.0 |
| 2025-03 | CT#107 | CP-250065 | 0010 | - | F | Correction in 15.xA.x test cases | 18.1.0 |
| 2025-03 | CT#107 | CP-250065 | 0011 | - | F | Correction to Unified Access Control Testcases 5.4.x | 18.1.0 |
| 2025-03 | CT#107 | CP-250065 | 0012 | - | B | Introduction of voice centric support for TS 31.127 5G tests | 18.1.0 |
| 2025-03 | CT#107 | CP-250065 | 0013 | - | B | UE recognising the search period of the Higher priority PLMN - NG-RAN | 18.1.0 |
| 2025-03 | CT#107 | CP-250065 | 0014 | - | B | Addition of SSIM test cases | 18.1.0 |
| 2025-03 | CT#107 | CP-250065 | 0015 | - | F | Correction to EFIMSI in clauses 4.3 and 4.6.x | 18.1.0 |
| 2025-03 | CT#107 | CP-250065 | 0016 | - | F | Correction to CSG TC 10.1.8 | 18.1.0 |
| 2025-06 | CT#108 | CP-251023 | 0017 | - | F | Correction to EFIMSI in clause 4.6.2 | 18.2.0 |
| 2025-06 | CT#108 | CP-251023 | 0018 | - | F | Correction to EF5GS3GPPLOCI for TC 7.4.7 | 18.2.0 |
| 2025-06 | CT#108 | CP-251023 | 0019 | - | F | Correction to EFEPSLOCI in clause 4.5.4 | 18.2.0 |
| 2025-06 | CT#108 | CP-251023 | 0020 | - | F | Correction to EFEPSLOCI for TC 10.1.2 | 18.2.0 |
| 2025-06 | CT#108 | CP-251023 | 0021 | - | F | Correction to default EF5GS3GPPNSC and EF5GSN3GPPNSC | 18.2.0 |
| 2025-06 | CT#108 | CP-251023 | 0022 | - | F | Correction to 5G AKA TC 15.2.5 | 18.2.0 |
| 2025-06 | CT#108 | CP-251023 | 0026 | - | F | Correction to SUCI calculation TC 5.3.16 | 18.2.0 |
| 2025-06 | CT#108 | CP-251023 | 0027 | - | D | Change reference from ETSI TS 102 223 to 3GPP TS 31.111 | 18.2.0 |
| 2025-06 | CT#108 | CP-251032 | 0024 | 3 | B | Example test environment for seamless testing | 18.2.0 |
| 2025-09 | CT#109 | CP-252019 | 0028 | - | F | Correction to SUCI calculation TC 5.3.17 | 18.3.0 |
| 2025-09 | CT#109 | CP-252019 | 0029 | - | F | Correction to SUCI calculation TC 5.3.12 | 18.3.0 |
| 2025-09 | CT#109 | CP-252019 | 0030 | - | F | Correction to UE identification TC 5.3.9 | 18.3.0 |
| 2025-09 | CT#109 | CP-252019 | 0031 | 2 | D | Clarification of Coding Conventions | 18.3.0 |
| 2025-09 | CT#109 | CP-252026 | 0032 | 2 | F | Missing figure in Clause D.1 | 18.3.0 |