| 3GPP TR 33.727 V0.0.1 (2025-07) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; LI Handling of Protected Services (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	9

3.1	Terms	9

3.2	Symbols	9

3.3	Abbreviations	10

4	Background	11

4.1	General	11

4.2	Architecture	11

4.2.1	Current LI Architecture	11

4.2.1.1	Overview	11

4.2.1.2	Roaming	12

4.2.1.3	LI Provisioning and Triggering	12

4.2.1.4	Generation of IRI	12

4.2.1.5	Generation of CC	13

4.3	Cryptographic processing for protected services	13

4.3.1	General	13

4.3.2	Handshake and cryptographic context initialization	14

4.3.3	Per-message processing	14

4.3.3.1	General	14

4.3.3.2	 Determination of CSI	14

4.3.3.3	 Determination of cryptographic keys	15

4.3.3.4	 Cryptographic message processing	15

4.3.4	Updating cryptographic context	16

4.4	Assumptions and principles	16

5	Key issues	17

5.1	Key issue: Cryptographic protocol considerations	17

5.1.1	Key issue #1.1: Security protocol detection	17

5.1.1.1	Key issue details	17

5.1.1.2	LI considerations	17

5.1.1.3	Potential LI requirements	17

5.1.2	Key issue #1.2: Obtaining key management IRI	17

5.1.2.1	Key issue details	17

5.1.2.2	LI considerations	17

5.1.3.3	Potential LI requirements	17

5.1.3	Key issue #1.3: Obtaining auxiliary security parameter IRI	17

5.1.3.1	Key issue details	17

5.1.3.2	LI considerations	17

5.1.3.3	Potential LI requirements	17

5.1.4	Key issue #1.4: Processing protected protocol PDUs	17

5.1.4.1	Key issue details	17

5.1.4.2	LI considerations	17

5.1.4.3	Potential LI requirements	17

5.1.5	Key issue #1.5: Processing encrypted handshake	18

5.1.5.1	Key issue details	18

5.1.5.2	LI considerations	18

5.1.5.3	Potential LI requirements	18

5.1.6	Key issue #1.6: Processing early data	18

5.1.6.1	Key issue details	18

5.1.6.2	LI considerations	18

5.1.6.3	Potential LI requirements	18

5.1.7	Key Issue #1.7: Unreliable transport and PDU Fragmentation	18

5.1.7.1	Key issue details	18

5.1.7.2	LI considerations	18

5.1.7.3	Potential LI requirements	18

5.1.8	Key issue #1.8: Perfect forward secrecy	18

5.1.8.1	Key issue details	18

5.1.8.2	LI considerations	18

5.1.8.3	Potential LI requirements	18

5.1.9	Key Issue #1.9: Session resumption	18

5.1.9.1	Key issue details	18

5.1.9.2	LI considerations	18

5.1.9.3	Potential LI requirements	18

5.2	Key issue: Mid-session intercept	19

5.2.1	Key issue #2.1: Handshake dependency	19

5.2.1.1	Key issue details	19

5.2.1.2	LI considerations	19

5.2.1.3	Potential LI requirements	19

5.2.2	Key issue #2.2: Obtaining cryptographic context synchronization	19

5.2.2.1	Key issue details	19

5.2.2.2	LI considerations	19

5.2.2.3	Potential LI requirements	19

5.3	Key issue: Roaming and trust model	19

5.3.1	Key issue #3.1: Inter-PLMN dependency	19

5.3.1.1	Key issue details	19

5.3.1.2	LI considerations	19

5.3.1.3	Potential LI requirements	19

5.3.2	Key issue #3.2: Separation of data confidentiality and integrity	19

5.3.2.1	Key issue details	19

5.3.2.2	LI considerations	19

5.3.2.3	Potential LI requirements	19

5.4	Key issue: LI architecture	20

5.4.1	Key issue #4.1: Decryption functionality	20

5.4.1.1	Key issue details	20

5.4.1.2	LI considerations	20

5.4.1.3	Potential LI requirements	20

5.4.2	Key issue #4.2: Provisioning and triggering	20

5.4.2.1	Key issue details	20

5.4.2.2	LI considerations	20

5.4.2.3	Potential LI requirements	20

5.5	Key issue: Security assurance	20

5.5.1	Key issue #5.1: Roaming interface security	20

5.5.1.1	Key issue details	20

5.5.1.2	LI considerations	20

5.5.1.3	Potential LI requirements	20

5.4.2	Key issue #5.2: Decryption functionality security	20

5.5.2.1	Key issue details	20

5.5.2.2	LI considerations	20

5.5.2.3	Potential LI requirements	20

5.6	Key issue: UE-to-UE protected communication	20

5.6.1	Key issue #5.3: Interception of UE-to-UE E2E protected communication	20

5.6.1.1	Key issue details	20

5.6.1.2	LI considerations	20

5.6.1.3	Potential LI requirements	20

6	Solutions	21

6.1	Mapping of solutions to key issues	21

6.2	Solutions for cryptographic protocol considerations	21

6.2.1	Solution #1.1: Security protocol detection	21

6.2.1.1	Introduction	21

6.2.1.2	Solution details	21

6.2.1.3	Evaluation	21

6.2.2	Solution #1.2: Use of AKMA (or equivalent)	21

6.2.2.1	Introduction	21

6.2.2.2	Solution details	21

6.2.2.3	Evaluation	21

6.2.3	Solution #1.3: Handshake interception	22

6.2.3.1	Introduction	22

6.2.3.2	Solution details	22

6.2.3.3	Evaluation	22

6.2.4	Solution #1.4: Security processing state machine	22

6.2.4.1	Introduction	22

6.2.4.2	Solution details	22

6.2.4.3	Evaluation	22

6.2.5	Solution #1.5: Cipher suite profiling	22

6.2.5.1	Introduction	22

6.2.5.2	Solution details	22

6.2.5.3	Evaluation	22

6.3	Solutions for mid-session intercept	22

6.3.1	Solution #2.1: Security state mirroring	22

6.3.1.1	Introduction	22

6.3.1.2	Solution details	22

6.3.1.3	Evaluation	22

6.4	Solutions for roaming	23

6.4.1	Solution #3.1: Extended AAnF and SEAF functionality	23

6.4.1.1	Introduction	23

6.4.1.2	Solution details	23

6.4.1.3	Evaluation	23

6.4.2	Solution #3.2: Key hierarchy and key separation	23

6.4.2.1	Introduction	23

6.4.2.2	Solution details	23

6.4.2.3	Evaluation	23

6.5	Solutions for LI architecture	23

6.5.1	Solution #4.1: Security State Mirroring Function (SSMF)	23

6.5.1.1	Introduction	23

6.5.1.2	Solution details	23

6.5.1.3	Evaluation	23

6.5.2	Solution #4.2: Decryption POI (D-POI)	23

6.5.2.1	Introduction	23

6.5.2.2	Solution details	23

6.5.2.3	Evaluation	23

6.5	Solutions for security assurance	24

6.6.1	Solution #5.1: D-POI SCAS	24

6.6.1.1	Introduction	24

6.6.1.2	Solution details	24

6.6.1.3	Evaluation	24

6.7	Solutions for UE-to-UE protected communication	24

6.7.1	Solution #5.2: Interception of UE-to-UE E2E protected communication	24

6.7.1.1	Introduction	24

6.7.1.2	Solution details	24

6.7.1.3	Evaluation	24

7	Conclusions	24

7.1	Impact of solutions	24

7.2	General conclusions	24

7.3	Conclusion on solution #1.1: Security protocol detection	24

7.4	Conclusion on solution #1.2: Use of AKMA (or equivalent)	24

Annex A (informative): Illustration of scenarios	25

A.1	Introduction	25

A.2	Use of AKMA with TLS 1.2	25

A.3	Use of AKMA with TLS 1.3	25

A.4	Use of AKMA with DTLS 1.3	25

Annex X: Change history	25

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

Offering communication services with end-to-end (E2E) cryptographic protection is becoming more desirable and more common. 3GPP-defined services (such as those of the IMS framework) have so far largely relied on trustworthy network infrastructure, complemented by strong cryptographic access security and hop-by-hop protection in the core and service domains. However, also here, E2E protected services are gaining attractivity. Indeed, the solution defined in TS 33.535, Authentication and Key Management for Applications based on 3GPP credentials in the 5G System (AKMA), can be leveraged as a basis for protecting basically any IP-based service provided to a UE (an ME with USIM-credentials). While this trend is both natural and desirable from a user privacy point of view, it creates problems related to regulatory obligations for CSPs to provide Lawful Interception (LI). The problems are particularly emphasised in roaming scenarios, where each of two different CSPs in distinct jurisdictions might independently need to provide LI without reliance on co-operation by the other CSP and/or law enforcement in the other jurisdiction.

The currently dominant approach to tackle these issues has been to simply not activate the protection (encryption) when it potentially conflicts with LI requirements. Going forward, this is most likely not a sustainable solution. For example, it potentially exposes the traffic to any 3rd party and, moreover, it typically also results in data integrity being disabled, while the issues are predominantly tied to data confidentiality.

The present document provides a technical study on how to enable protected services without negative consequences for LI. The CSP’s LI obligation to provide communication with the encryption removed only applies when the CSP also provides the technical means enabling the protection (e.g. key management), and therefore the scope is limited to that use case.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.126: "Lawful Interception requirements".

[3]	3GPP TS 33.127: "Lawful Interception (LI) architecture and functions".

[4]	3GPP TS 33.128: "Protocol and procedures for Lawful Interception (LI)".

[5]	3GPP TS 33.163: "Battery Efficient Security for very low throughput Machine Type Communication (MTC) devices (BEST)".

[6]	3GPP TS 33.203: "3G security; Access security for IP-based services".

[7]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[8] 	3GPP TS 33.222:	 "Generic Authentication Architecture (GAA); Access to network application functions using Hypertext Transfer Protocol over Transport Layer Security (HTTPS)"

[9]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[10]	3GPP TS 33.535: "Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)".

[11]	3GPP TR 33.828: "IP Multimedia Subsystem (IMS) media plane security".

[12]	RFC 3711, "The Secure Real-time Transport Protocol (SRTP)", IETF.

[13]	RFC 5246, "The Transport Layer Security (TLS) Protocol Version 1.2", IETF.

[14]	RFC 8446, "The Transport Layer Security (TLS) Protocol Version 1.3", IETF.

[15]	RFC 9147, "The Datagram Transport Layer Security (DTLS) Protocol Version 1.3", IETF.

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

The study of the present document is concerned with providing a technical description of how to process, as part of Lawful Interception, a flow of security protected PDUs, transmitted across a CSP network, where said PDUs carry UP traffic related to some service. Specifically, the processing is carried out at a point inside the CSP network, using cryptographic transforms such as decryption, which would otherwise only take place at the receiving endpoint. To make the description of this processing unambiguous and as simple as possible, the following terms are defined.

Cryptographic context Information maintained locally at the sender/receiver and used as basis to determine how to cryptographically process each PDU. This information includes cryptographic keys, information about which cryptographic algorithms to use, (partial) cryptographic synchronization information, etc.

Cryptographic Synchronization Information (CSI) The complete set of information used (together with cryptographic keys) as input to the cryptographic processing of each PDU at the sender/receiver. CSI is formed by a combination of information from the cryptographic context and explicit cryptographic synchronization information (see below).

Decryption Point-of-Intercept (D-POI) A CC-POI inside the CSP network which processes encrypted xCC, decrypting it to produce plaintext xCC.

Explicit Cryptographic Synchronization Information (ECSI) Part of CSI carried in-band, as part of, or derived from, security protocol PDUs.

Security protocol A protocol used between a sender and a receiver to provide security services such as data confidentiality and/or data integrity to communication. The security protocol receives an SDU from higher layer and produce security protocol PDUs, transmitted over lower layers. In the present document, the sender and receiver roles typically correspond to a UE and some application layer service, respectively. However, it is also possible that both the sender and receiver are UEs.

Perfect forward secrecy Property of a security protocol by which the compromise of keys involved in a particular session does not negatively affect the security of prior sessions. If the compromised keys are only of temporary nature, also future sessions are protected from adverse effects.

## 3.2	Symbols

Void

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AAnF	AKMA Anchor Function

ADMF	Administration Function

AF	Application Function

AKID	AKMA Key Identifier

AKMA	Authentication and Key Management for Applications

CC	Content of Communication

CID	Connection Identifier

CSI	Cryptographic Synchronization Information

CSP	Communication Service Provider

DN	Data Network

D-POI	Decryption Point of Intercept

E2E	End-to-end

E2EE				End-to-end Encryption

ECSI	Explicit Cryptographic Synchronization Information

FQDN	Fully Qualified Domain Name

GBA	Generic Bootstrapping Architecture

HPLMN	Home PLMN

IMS	IP Multimedia Subsystem

IRI	Intercept Related Information

IV	(cryptographic) Initialization Value

KID	AKMA Key Identifier

KLI	Decryption key(s) for services encrypted by CSP-provided keys

KSF	Key Server Function

LEA	Law Enforcement Agency

LEMF	Law Enforcement Monitoring Facility

LI	Lawful Interception

LICF	Lawful Interception Control Function

LI_HI1	LI_Handover Interface 1

LI_HI2	LI_Handover Interface 2

LI_HI3	LI_Handover Interface 3

LIPF	LI Provisioning Function

LI_T1	Lawful Interception Triggering Interface 1

LI_T3	Lawful Interception Triggering Interface 3

LI_X1	Lawful Interception Internal Interface 1

LI_X2	Lawful Interception Internal Interface 2

LI_X3	Lawful Interception Internal Interface 3

MAC	Message Authentication Code

MDF	Mediation and Delivery Function

MDF2	Mediation and Delivery Function 2

MDF3	Mediation and Delivery Function 3

NF	Network Function

PDU	Protocol Data Unit

PLMN	Public Land Mobile Network

POI	Point Of Interception

PSK	Pre-Shared Key

SCAS	Security Assurance Specification

SDU	Service Data Unit

SMF	Session Management Function

SRTP	Secure Real-time Transport Protocol

STF	Security Terminating Function

SSMF	Security-state Mirroring Function

SUPI	Subscriber Permanent Identifier

TF	Triggering Function

UE	User Equipment

UPF	User Plane Function

VPLMN	Visited PLMN

xCC	LI_X3 Communications Content

xIRI	LI_X2 Intercept Related Information

# 4	Background

## 4.1	General

3GPP has currently defined two general frameworks that allow subscriber credentials (the USIM) to be utilized to derive cryptographic keys for essentially any type of IP-based service. The two frameworks are the Generic Bootstrapping Architecture (GBA, TS 33.220 [7]) and Authentication and Key Management for Applications based on 3GPP credentials in the 5G System (AKMA, TS 33.535 [10]). There also exists service-specific security solutions that rely on USIM, e.g. the Battery Efficient Security for very low throughput Machine Type Communication devices (BEST, TS 33.163 [5]) and the Access security for IP-based services (IMS signalling security, TS 33.203 [6]), the latter with media plane security extensions described in TR 33.828 [11].

In all of these cases, a situation can occur resulting in that traffic is protected (including encryption) in a more network-wide fashion (potentially end-to-end), in turn potentially leading to conflicts with fulfilling regulatory obligations with respect to Lawful Interception. If a CSP initiates encoding, compression, or encryption of telecommunications traffic, LEAs generally require the CSP to provide intercepted communications "en clair". Such requirements are observed in the 3GPP specifications, but until recently, mainly by expressing recommendations of the use of encryption.

EXAMPLE:	As stated in [5], Annex A: "Due to regulatory requirements, operators may have to disable the BEST service for UEs roaming in their network".

This approach is not optimal since it in practice leads to disabling of the traffic protection altogether (including data encryption, data integrity, and data origin authentication).

## 4.2	Current LI Architecture

### 4.2.1	Overview

As of 3GPP Release 17, the Lawful Interception architecture of TS 33.127 has been extended by normative specifications describing a general LI-framework for services encrypted by CSP-provided keys and a more specific solution for the case that the service is protected (encrypted) based on AKMA keys (see clause 7.15 of [3]).

To this end, the architectural framework of TS 33.127 [3] defines the notion of a Security Termination Function (STF). This corresponds to the NF providing some service to the UE (messaging, chat, etc) and where the service is encrypted by CSP-provided keys. The encryption is applied end-to-end, between the UE and the STF. The CSP-provided keys, in turn, are provided by a Key Server Function (KSF).

EXAMPLE: A concrete instantiation of an STF and KSF can be exemplified with the KSF corresponding to an AKMA Anchor Function (AAnF) and the STF corresponding to an AKMA Application Function (AF) according to TS 33.535 [10].

Figure 4.2.1-1: General (non-roaming) LI-architecture for CSP-provided keys according to TS 33.127. If the STF uses 5G-native identifiers, the triggering of POIs in the STF from the IRI-TF and CC-TF in the KSF could be replaced by direct provisioning of the STF using LI_X1, see text.

### 4.2.2	Roaming

The current LI-architecture of TS 33.127 covers only the non-roaming case.

### 4.2.3	LI Provisioning and Triggering

The KSF is provisioned based on SUPI. If also the STF provides services based on a 5G-native identifier such as SUPI, standard LI provisioning over LI_X1 is possible also for the STF. If some other identifier is used by the STF, it might be necessary to instead use triggering: determination of a subscriber as being an LI target might not be possible until the STF actively requests key material from the KSF: triggering functions of the KSF maps the request to a SUPI and triggers the STF.

### 4.2.4	Generation of IRI

The scope of the LI-solution defined in TS 33.127 is to collect IRI from the KSF (e.g. CSP-provided keys) and optionally from the STF over LI_X2. IRI collected from the STF can comprise further keys and/or other auxiliary security parameters such as the selected encryption protocol or algorithm.

The IRI collected from the KSF comprises at least key material which forms the basis for the actual encryption/decryption of the services, between the STF and a UE. This key material is generically referred to as KLI. The key KLI is associated with a specific subscriber and could also be tied to a specific (subscriber, STF)-pair. In the latter case, KLI is derived from a user-specific key KUSER. There is an identifier associated with KUSER which uniquely identifies the KUSER, at least within the CSP and this identifier is generically denoted KID.

When the UE requests protected services from an STF, the request typically starts with a security handshake, establishing further IRI related to the protection. For key management protocols such as AKMA, the STF might not always be located within the CSP domain. Thus, in order to make the discussion as generic as possible, the present document makes no assumption on the ability to collect any IRI whatsoever from the STF. As a consequence, collection of IRI from in-band signalling between UE and STF becomes an identified key issue (see clause 5.1.3).

If AKMA is used, a mapping from TS 33.127 to TS 33.535 can be made according to table 4.2.4-1.

Table 4.2.4-1: Mapping functions between the general architecture and AKMA

| Term in the present document and TS 33.127 | AKMA correspondence | Reference |
| --- | --- | --- |
| KSF | AAnF | TS 33.535 [10] clause 4.2.1 |
| STF | AF | TS 33.535 [10] clause 4.2.2 |
| KUSER | KAKMA | TS 33.535 [10] clause 6.1 |
| KSTF | KAF | TS 33.535 [10] clause 6.2 |
| KLI | KAKMA and/or KAF | TS 33.535 [10] clause 6.1, 6.2 |
| Key identifier, KID | A-KID | TS 33.535 [10] clause 4.4.2 |
| auxiliary security parameters | Ua* security protocol parameters | TS 33.535 [10] clause 4.4.1 |

NOTE 1:	In almost all cases, the encryption/decryption key used is distinct in dependence on whether the traffic is sent from UE to the STF, or, from STF to UE. Both of these keys are usually derived from the same KLI and appropriate notation will be introduced later to unambiguously refer to the two directional keys when this is relevant.

NOTE 2:	For the purposes of the present document, the security protocol applied to UP traffic between the STF and UE can be assumed to use symmetric cryptography so that the terms "encryption key" and "decryption key" refer to one and the same key.

### 4.2.5	Generation of CC

In case the STF provides a 3GPP-defined service operated by the CSP, the STF might further be equipped with a CC-POI. If this is the case, there is in general no need to use the collected IRI to explicitly perform decryption of the LI product since a CC-POI can then be assumed to reside "behind" the decryption functionality of the STF. However, since the present document makes no assumption on LI functionality at the STF, this case is not discussed further. Instead, defining an appropriate CC-POI functionality within the CSP network is defined as one of the key issues (see clause 5.4.1).

## 4.3	Cryptographic processing for protected services

### 4.3.1	General

The processing that needs to be performed within the CSP's LI system to convert protected (encrypted) user plane PDUs into plaintext service-related communication content is for obvious reasons very similar to the processing that takes place at the receiving end-point. This, in turn, mirrors processing at the sender.

EXAMPLE:	An encryption processing step at the sender will correspond to a decryption processing step at the receiver.

Therefore, to understand the problem space better, it is useful to first describe the security processing at the sender/receiver in terms of a hypothetical security protocol which describes the processing steps as generally as possible. Readers well acquainted with security protocols can without loss skip through the remainder of this clause.

All security protocols have a quite universally valid structure in terms of processing flow.

Figure 4.3.1-1: Security protocol processing overview.

### 4.3.2	Handshake and cryptographic context initialization

Most protocols first use a handshake phase which is used to (mutually) authenticate the communicating parties, and to initiate a cryptographic context to be used to protect a specific session. This involves creating the necessary session keys, negotiating which cryptographic algorithms to use, etc. In general, the handshake results in initializing a local cryptographic context (or state) at the sender and receiver. When CSP-provided key management is used, the CSP-provided keys can be used for authentication and the session keys are then created in dependence of those CSP-provided keys. Whether such handshake occurs depends on whether the security protocol has integrated its own authentication and key management functionality or whether a separate authentication and key agreement protocol is used for that purpose.

EXAMPLE:	The (D)TLS protocols [13,14,15] have the authentication and key management "built-in" as discussed above, while the Secure Real-time Transport Protocol (SRTP) [12] does not.

The handshake is often a "once-per-session" matter, though further handshake messages might occur during an established session, e.g. to update keys.

### 4.3.3	Per-message processing

#### 4.3.3.1	General

After the handshake has been completed, the security protocol handles its main responsibility. At the receiver end, this corresponds to converting received security protocol PDUs into plaintext messages passed upward in the stack to the application. This per-message processing is the focus of the present clause and a process flow is shown in figure 4.3.3.1-1 below.

Figure 4.3.3.1-1: Security protocol processing (per message) on the receiving end.

NOTE:	Since the present document focuses on the decryption process at within the CSP LI-system, processing at the sender end is largely omitted from the discussion.

The step "cryptographic processing" can be broken down into sub-steps "decryption" and "integrity verification" but the order of those two sub-steps can sometimes be reversed and the two sub-steps can sometimes be integrated into a single cryptographic transform.  The steps are described in more detail below.

#### 4.3.3.2		Determination of CSI

In order to cope with possible unreliability of lower protocol layers (e.g. packet loss or reorder) and also to protect against malicious interference by an adversary, e.g. attempting to replay protected messages, each security protocol message (PDU) is processed relative to cryptographic synchronization information (CSI). This information can in part be carried in-band, denoted explicit cryptographic synchronization information (ECSI) and part of it can be implicit, as part of, or derived from, the cryptographic context. The CSI serves the purpose of assigning a unique "identity" to each message/packet, allowing the receiver to reject replayed messages and to determine the position (ordering) of the message within the session. It is common to base the CSI on a sequence number or the like. This sequence number (or part thereof) is typically carried in-band when unreliable transport is used and can otherwise be part of the cryptographic context.  For cryptographic reasons, another part of the CSI is often a random value which can be fixed for a session and/or be unique to each PDU.

EXAMPLE:	Since TLS is likely to be commonly used, it is of interest to exemplify the CSI using this protocol. The CSI for decryption used in TLS always comprises an ECSI carried in the TLS record payload. For certain cipher suites (so called AEAD-algorithms, see clause 4.2.3.4) the CSI additionally makes use of an implicit component in the form of a per-session "nonce" (obtained from the handshake) and the 64-bit TLS sequence number (the two latter being part of the cryptographic context). The integrity verification in TLS always makes use of the 64-bit TLS sequence number for replay protection. Because TLS runs over TCP, there is no need to explicitly notify the TLS-layer of the next sequence number.

#### 4.3.3.3		Determination of cryptographic keys

In most cases, the cryptographic keys used as input to cryptographic processing are static for the duration of the communication session and are stored as part of the cryptographic context. However, some notable examples to the contrary exist.

EXAMPLE 1:	The SRTP protocol [12] has an option to deterministically refresh keys during a session: new keys are created after every r:th processed SRTP packet (r is a configurable parameter).

EXAMPLE 2:	The (D)TLS 1.3 protocols [14,15] can start to transmit encrypted application data even before the handshake is complete and parts of the handshake can also be encrypted. In these cases, special keys are used for the "early data" and the handshake. New set of keys are taken into use after the handshake completes. These protocols can also update keys at any point in time by a special key update message.

#### 4.3.3.4		Cryptographic message processing

As mentioned, the security protocol typically employs both data encryption and data integrity.

NOTE:	From LI point of view, security protocols providing data integrity provides stronger means of attributing the intercepted traffic to a target's communication as only the target UE, the STF (or the CSP itself) could generate traffic that verifies correctly by knowledge of the keys. Integrity protection can also aid mid-session intercept as will be elaborated later in the present document.

Therefore, while the main focus of LI can be argued to be that of acquiring plaintext xCC, verifying integrity in the LI-system also brings benefits to the overall LI-product.

The integrity protection comprises adding a message authentication code (MAC) at the sender and to verify this at the receiver. To this end, there are four possibilities:

- The security protocol first encrypts the application SDU, then adds integrity protection of the encrypted SDU (encrypt-then-MAC).

- The security protocol encrypts the application SDU, but adds integrity protection for the original unencrypted (plaintext) SDU (encrypt-and-MAC).

- The security protocol adds integrity protection to the plaintext SDU, then encrypts both the plaintext SDU and the added message authentication code (MAC-then-encrypt).

- A so called authenticated encryption algorithm (AEAD) is used, which bundles data encryption with data integrity in (more or less) a single cryptographic transformation.

This maps to corresponding processing at the receiving end. In the first three cases, separate cryptographic keys are typically needed for encryption and data integrity while in the fourth case, a single key can be used. In the concrete examples based on the TLS-protocol described later in the present document, only approaches 3 and 4 occur.

As discussed, the communicating parties are also interested in protecting against replayed messages. This is logically part of the integrity verification step. To this end, the CSI is used. The CSI therefore, at least in part, often takes the form of a sequence number, incrementing by "+1" on each message. When the receiver starts to process a new message, it first checks if a PDU with that sequence number has already been received (and verified). If so, the PDU is discarded as a potential replayed message. Otherwise, the integrity verification is performed. If the integrity verifies correctly, the receiver locally marks the corresponding message counter as received, and otherwise discards the message. Depending on whether reliable transport is used or not, the replay protection scheme might need to be complemented with more elaborate window-based mechanisms, accepting messages whose sequence numbers lie within a certain window, relative to the previously received messages. Information about the window is stored in the cryptographic context. A more in-depth discussion is provided in annex A.

### 4.3.4	Updating cryptographic context

This step comprises updating the information in the cryptographic context in preparation for the next PDU. For example, updating the window of received PDUs, updating information relating to keys and/or CSI, etc.

## 4.4	Assumptions and principles

For the purpose of the present document, the following assumptions are made, in particular assumptions about the principal properties of the desired LI functionality. Examples and notes clarifying the rationale and impact of the assumptions and principles are also provided.

A0. Unless otherwise noted, the remainder of the present document uses the term "the CSP" to refer to the CSP for which LI obligations apply.

A1. The STF does not reside within the CSP and is not directly usable to collect further IRI.

EXAMPLE 1:	In a non-roaming case, the STF can be located at an external DN.

EXAMPLE 2:	In a roaming case, the STF could be located in the HPLMN or at an external DN.

A2. The key management is based on USIM (in the roaming case administrated by the home CSP) and follows some 3GPP specification.

A3. The LI solution needs to have the capability of at least decrypting the UE-STF traffic.

A4. The LI solution is autonomous and does not depend on inter-CSP, LI-specific assistance.

NOTE:	In the roaming case, full information on AKMA keys is in general only available in the HPLMN.

A5. The security (encryption) protocol in use between UE and STF also follows 3GPP specifications (possibly by cross-referencing a specification of some other SDO).

EXAMPLE:		The CSP-provided keys alone could be insufficient to perform decryption, due to that the actual decryption key(s) depend also on protocol-specific signaling exchanged inband between the UE and the STF. The CSP networks needs the capability to detect and recognize this signaling.

A6. The function within the CSP's LI-system performing security processing (decryption), receives protected PDUs with the same or better quality than that of the PDUs arriving at the service endpoints (UE or STF).

EXAMPLE:	The term "quality" could refer to PDU bit-errors, loss, or amount of reordering.

NOTE:	A6 does not imply an assumption about reliable transport. It is fully within the scope to study handling of services transported over e.g. UDP.

A7. The LI product by the CSP needs to be complete and avoiding under-collection.

EXAMPLE: Start of intercept with already established session is to be supported (mid-session intercept).

A8. It is desirable that any proposed solution also works for services which are end-to-end protected between two UEs and using CSP-provided keys, as long as this is feasible without undue security or complexity impact.

NOTE:	This is a consequence of A1. Assumption A1 makes it necessary for any solution to base its LI-functionality solely on the knowledge of the CSP-provided keys and inband signalling traversing the CSP network.

# 5	Key issues

Editor's note: The initial list of key issues has been derived mainly from T-doc s3i250216 presented at SA3LI#97.

## 5.1	Key issue: Cryptographic protocol considerations

### 5.1.1	Key issue #1.1: Security protocol detection

#### 5.1.1.1	Key issue details

#### 5.1.1.2	LI considerations

#### 5.1.1.3	Potential LI requirements

### 5.1.2	Key issue #1.2: Obtaining key management IRI

#### 5.1.2.1	Key issue details

#### 5.1.2.2	LI considerations

#### 5.1.3.3	Potential LI requirements

### 5.1.3	Key issue #1.3: Obtaining auxiliary security parameter IRI

#### 5.1.3.1	Key issue details

#### 5.1.3.2	LI considerations

#### 5.1.3.3	Potential LI requirements

### 5.1.4	Key issue #1.4: Processing protected protocol PDUs

#### 5.1.4.1	Key issue details

#### 5.1.4.2	LI considerations

#### 5.1.4.3	Potential LI requirements

### 5.1.5	Key issue #1.5: Processing encrypted handshake

#### 5.1.5.1	Key issue details.

#### 5.1.5.2	LI considerations

#### 5.1.5.3	Potential LI requirements

### 5.1.6	Key issue #1.6: Processing early data

#### 5.1.6.1	Key issue details

#### 5.1.6.2	LI considerations

#### 5.1.6.3	Potential LI requirements

### 5.1.7	Key Issue #1.7: Unreliable transport and PDU Fragmentation

#### 5.1.7.1	Key issue details

#### 5.1.7.2	LI considerations

#### 5.1.7.3	Potential LI requirements

### 5.1.8	Key issue #1.8: Perfect forward secrecy

#### 5.1.8.1	Key issue details

#### 5.1.8.2	LI considerations

#### 5.1.8.3	Potential LI requirements

### 5.1.9	Key Issue #1.9: Session resumption

#### 5.1.9.1	Key issue details

#### 5.1.9.2	LI considerations

#### 5.1.9.3	Potential LI requirements

## 5.2	Key issue: Mid-session intercept

### 5.2.1	Key issue #2.1: Handshake dependency

#### 5.2.1.1	Key issue details

#### 5.2.1.2	LI considerations

#### 5.2.1.3	Potential LI requirements

### 5.2.2	Key issue #2.2: Obtaining cryptographic context synchronization

#### 5.2.2.1	Key issue details

#### 5.2.2.2	LI considerations

#### 5.2.2.3	Potential LI requirements

## 5.3	Key issue: Roaming and trust model

### 5.3.1	Key issue #3.1: Inter-PLMN dependency

#### 5.3.1.1	Key issue details

#### 5.3.1.2	LI considerations

#### 5.3.1.3	Potential LI requirements

### 5.3.2	Key issue #3.2: Separation of data confidentiality and integrity

#### 5.3.2.1	Key issue details

#### 5.3.2.2	LI considerations

#### 5.3.2.3	Potential LI requirements

## 5.4	Key issue: LI architecture

### 5.4.1	Key issue #4.1: Decryption functionality

#### 5.4.1.1	Key issue details

#### 5.4.1.2	LI considerations

#### 5.4.1.3	Potential LI requirements

### 5.4.2	Key issue #4.2: Provisioning and triggering

#### 5.4.2.1	Key issue details

#### 5.4.2.2	LI considerations

#### 5.4.2.3	Potential LI requirements

## 5.5	Key issue: Security assurance

### 5.5.1	Key issue #5.1: Roaming interface security

#### 5.5.1.1	Key issue details

#### 5.5.1.2	LI considerations

#### 5.5.1.3	Potential LI requirements

### 5.4.2	Key issue #5.2: Decryption functionality security

#### 5.5.2.1	Key issue details

#### 5.5.2.2	LI considerations

#### 5.5.2.3	Potential LI requirements

## 5.6	Key issue: UE-to-UE protected communication

### 5.6.1	Key issue #6.1: Interception of UE-to-UE E2E protected communication

#### 5.6.1.1	Key issue details

#### 5.6.1.2	LI considerations

#### 5.6.1.3	Potential LI requirements

# 6	Solutions

## 6.1	Mapping of solutions to key issues

Table 6.1-1: Mapping of solutions to key issues

| Solution | Key Issue |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.1 | 1.2 | 1.3 | 1.4 | 1.5 | 1.6 | 1.7 | 1.8 | 1.9 | 2.1 | 2.2 | 3.1 | 3.2 | 4.1 | 4.2 | 5.1 | 6.1 |
| 1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## 6.2	Solutions for cryptographic protocol considerations

### 6.2.1	Solution #1.1: Security protocol detection

Editor’s note: aimed at addressing KI 1.1 Security protocol detection

#### 6.2.1.1	Introduction

#### 6.2.1.2	Solution details

#### 6.2.1.3	Evaluation

### 6.2.2	Solution #1.2: Use of AKMA (or equivalent)

Editor’s note: aimed at addressing KI 1.2 Obtaining key management IRI

#### 6.2.2.1	Introduction

#### 6.2.2.2	Solution details

#### 6.2.2.3	Evaluation

### 6.2.3	Solution #1.3: Handshake interception

Editor’s note: aimed at addressing KI 1.3 Obtaining auxiliary security parameter IRI

#### 6.2.3.1	Introduction

#### 6.2.3.2	Solution details

#### 6.2.3.3	Evaluation

### 6.2.4	Solution #1.4: Security processing state machine

Editor’s note: aimed at addressing KI 1.4 Processing protected protocol PDUs, 1.5 Processing encrypted handshake, 1.6 Processing early data, and 1.7 Unreliable transport and protocol PDU fragmentation

#### 6.2.4.1	Introduction

#### 6.2.4.2	Solution details

#### 6.2.4.3	Evaluation

### 6.2.5	Solution #1.5: Cipher suite profiling

Editor’s note: aimed at addressing KI 1.8 PFS and 1.9 Session resumption

#### 6.2.5.1	Introduction

#### 6.2.5.2	Solution details

#### 6.2.5.3	Evaluation

## 6.3	Solutions for mid-session intercept

Editor’s note: aimed at addressing KI 2.1 Handshake dependency and 2.2 Obtaining cryptographic context synchronization

### 6.3.1	Solution #2.1: Security state mirroring

#### 6.3.1.1	Introduction

#### 6.3.1.2	Solution details

#### 6.3.1.3	Evaluation

## 6.4	Solutions for roaming

### 6.4.1	Solution #3.1: Extended AAnF and SEAF functionality

Editor’s note: aimed at addressing KI 3.1 Inter-PLMN dependency and 5.1 Roaming interface security

#### 6.4.1.1	Introduction

#### 6.4.1.2	Solution details

#### 6.4.1.3	Evaluation

### 6.4.2	Solution #3.2: Key hierarchy and key separation

Editor’s note: aimed at addressing KI 3.2 Separation of data confidentiality and integrity, 5.1 roaming interface security and 5.2 Decryption functionality security

#### 6.4.2.1	Introduction

#### 6.4.2.2	Solution details

#### 6.4.2.3	Evaluation

## 6.5	Solutions for LI architecture

### 6.5.1	Solution #4.1: Security State Mirroring Function (SSMF)

Editor’s note:	aimed at addressing KI 2.1 Handshake dependency, 2.2 Obtaining cryptographic context synchronization, and 4.2 Provisioning and triggering

#### 6.5.1.1	Introduction

#### 6.5.1.2	Solution details

#### 6.5.1.3	Evaluation

### 6.5.2	Solution #4.2: Decryption POI (D-POI)

Editor’s note: aimed at addressing KI 4.1 Decryption functionality and 4.2 Provisioning and triggering

#### 6.5.2.1	Introduction

#### 6.5.2.2	Solution details

#### 6.5.2.3	Evaluation

## 6.5	Solutions for security assurance

### 6.6.1	Solution #5.1: D-POI SCAS

Editor’s note: aimed at addressing KI 5.2 Decryption functionality security

#### 6.6.1.1	Introduction

#### 6.6.1.2	Solution details

#### 6.6.1.3	Evaluation

## 6.7	Solutions for UE-to-UE protected communication

### 6.7.1	Solution #6.1: Interception of UE-to-UE E2E protected communication

Editor’s note: aimed at addressing KI 5.3 Interception of UE-to-UE E2E protected communication

#### 6.7.1.1	Introduction

#### 6.7.1.2	Solution details

#### 6.7.1.3	Evaluation

# 7	Conclusions

## 7.1	Impact of solutions

## 7.2	General conclusions

## 7.3	Conclusion on solution #1.1: Security protocol detection

## 7.4	Conclusion on solution #1.2: Use of AKMA (or equivalent)

Editor’s note: To be continued.

Annex A (informative):
Illustration of scenarios

## A.1	Introduction

For better understanding of the technical problems arising with implementing the solution concepts considered in clause 6, some concrete scenarios for the case that AKMA is used to handle CSP-provided keys, together with various flavors of the TLS/DTLS protocols [13,14,15]. This is motivated by the fact that the AKMA framework is the main solution available in 3GPP specifications and the fact that TLS/DTLS are the main protocols for which AKMA-specific profiles are defined.

Editor's note: this annex is intended to be based on prior T-doc S3i250126 (clause 6.2), presented at SA3LI#97.

## A.2	Use of AKMA with TLS 1.2

## A.3	Use of AKMA with TLS 1.3

## A.4	Use of AKMA with DTLS 1.3

Annex X:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-07 | SA3#98-LI | S3-250360 |  |  |  | TR skeleton and initial draft | 0.0.1 |