3GPP TS 36.211 V17.3.0 (2023-03)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Evolved Universal Terrestrial Radio Access (E-UTRA);

Physical channels and modulation

(Release 17)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

E-UTRA, radio, layer 1

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	9

1	Scope	10

2	References	10

3	Symbols and abbreviations	11

3.1	Symbols	11

3.2	Abbreviations	15

4	Frame structure	15

4.1	Frame structure type 1	16

4.2	Frame structure type 2	17

4.3	Frame structure type 3	18

5	Uplink	20

5.1	Overview	20

5.1.1	Physical channels	20

5.1.2	Physical signals	20

5.2	Slot structure and physical resources	20

5.2.1	Resource grid	20

5.2.2	Resource elements	22

5.2.3	Resource blocks	22

5.2.4	Narrowbands and widebands	22

5.2.5	Guard period for narrowband and wideband retuning	23

5.3	Physical uplink shared channel	25

5.3.1	Scrambling	25

5.3.2	Modulation	26

5.3.2A	Layer mapping	27

5.3.2A.1	Layer mapping for transmission on a single antenna port	27

5.3.2A.2	Layer mapping for spatial multiplexing	27

5.3.3	Transform precoding	28

5.3.3A	Precoding	28

5.3.3A.1	Precoding for transmission on a single antenna port	28

5.3.3A.2	Precoding for spatial multiplexing	28

5.3.4	Mapping to physical resources	31

5.4	Physical uplink control channel	37

5.4.1	PUCCH formats 1, 1a and 1b	38

5.4.2	PUCCH formats 2, 2a and 2b	41

5.4.2A	PUCCH format 3	42

5.4.2B	PUCCH format 4	44

5.4.2C	PUCCH format 5	44

5.4.3	Mapping to physical resources	45

5.4A	Short Physical Uplink Control Channel	48

5.4A.1	General	48

5.4A.2	SPUCCH formats 1,1a,1b	48

5.4A.2.1	Slot-SPUCCH	48

5.4A.2.2	Subslot-SPUCCH	49

5.4A.3	SPUCCH format 3	50

5.4A.3.1	Slot-SPUCCH	50

5.4A.4	SPUCCH format 4	50

5.4A.4.1	Slot-SPUCCH	50

5.4A.4.2	Subslot-SPUCCH	51

5.4A.5	Mapping to physical resources	51

5.5	Reference signals	55

5.5.1	Generation of the reference signal sequence	55

5.5.1.1	Base sequences of length  or larger	56

5.5.1.2	Base sequences of length less than 	57

5.5.1.3	Group hopping	61

5.5.1.4	Sequence hopping	62

5.5.1.5	Determining virtual cell identity for sequence generation	62

5.5.2	Demodulation reference signal	63

5.5.2.1	Demodulation reference signal for PUSCH	63

5.5.2.1.1	Reference signal sequence	63

5.5.2.1.2	Mapping to physical resources	67

5.5.2.1A	Demodulation reference signal for PUSCH with sub-PRB allocations	68

5.5.2.1A.1	Reference signal sequence using modulation schemes other than π/2-BPSK	68

5.5.2.1A.2	Reference signal sequence using π/2-BPSK modulation scheme	69

5.5.2.1A.3	Group hopping	70

5.5.2.1A.4	Mapping to physical resources	71

5.5.2.2	Demodulation reference signal for PUCCH	71

5.5.2.2.1	Reference signal sequence	71

5.5.2.2.2	Mapping to physical resources	73

5.5.2.3	Demodulation reference signal for SPUCCH	74

5.5.2.3.1	Reference signal sequence	74

5.5.2.3.2	Mapping to physical resources	74

5.5.3	Sounding reference signal	76

5.5.3.1	Sequence generation	76

5.5.3.1.1	Sequence generation for basic SRS	76

5.5.3.1.2	Sequence generation for additional SRS	76

5.5.3.2	Mapping to physical resources	77

5.5.3.2.1	Mapping to physical resources for basic SRS	77

5.5.3.2.2	Mapping to physical resources for additional SRS	79

5.5.3.3	Sounding reference signal subframe configuration	80

5.6	SC-FDMA baseband signal generation	82

5.6A	SC-FDMA baseband signal generation for PUSCH using sub-PRB allocations	83

5.6A.1	Modulation schemes other than π/2-BPSK	83

5.6A.2	Modulation scheme π/2-BPSK	83

5.7	Physical random access channel	84

5.7.1	Time and frequency structure	84

5.7.2	Preamble sequence generation	91

5.7.3	Baseband signal generation	96

5.8	Modulation and upconversion	97

6	Downlink	98

6.1	Overview	98

6.1.1	Physical channels	98

6.1.2	Physical signals	98

6.2	Slot structure and physical resource elements	99

6.2.1	Resource grid	99

6.2.2	Resource elements	100

6.2.3	Resource blocks	101

6.2.3.1	Virtual resource blocks of localized type	102

6.2.3.2	Virtual resource blocks of distributed type	102

6.2.4	Resource-element groups (REGs)	103

6.2.4A	Enhanced Resource-Element Groups (EREGs)	104

6.2.4B	Short Resource-Element Groups (SREGs)	104

6.2.5	Guard period for half-duplex FDD operation	104

6.2.6	Guard Period for TDD Operation	105

6.2.7	Narrowbands and widebands	105

6.2.8	Guard period for narrowband and wideband retuning	106

6.3	General structure for downlink physical channels	106

6.3.1	Scrambling	107

6.3.2	Modulation	108

6.3.3	Layer mapping	108

6.3.3.1	Layer mapping for transmission on a single antenna port	108

6.3.3.2	Layer mapping for spatial multiplexing	109

6.3.3.3	Layer mapping for transmit diversity	110

6.3.4	Precoding	110

6.3.4.1	Precoding for transmission on a single antenna port	110

6.3.4.2	Precoding for spatial multiplexing using antenna ports with cell-specific reference signals	111

6.3.4.2.1	Precoding without CDD	111

6.3.4.2.2	Precoding for large delay CDD	111

6.3.4.2.3	Codebook for precoding and CSI reporting	112

6.3.4.3	Precoding for transmit diversity	113

6.3.4.4	Precoding for spatial multiplexing using antenna ports with UE-specific reference signals	114

6.3.5	Mapping to resource elements	115

6.4	Physical downlink shared channel	116

6.4.1	Physical downlink shared channel for BL/CE UEs	118

6.4.2	Slot/subslot-based physical downlink shared channel	121

6.5	Physical multicast channel	123

6.6	Physical broadcast channel	123

6.6.1	Scrambling	123

6.6.2	Modulation	123

6.6.3	Layer mapping and precoding	124

6.6.4	Mapping to resource elements	124

6.6.4.1	PBCH repetition in the cell acquisition subframe	125

6.7	Physical control format indicator channel	126

6.7.1	Scrambling	126

6.7.2	Modulation	126

6.7.3	Layer mapping and precoding	127

6.7.4	Mapping to resource elements	127

6.8	Physical downlink control channel	127

6.8.1	PDCCH formats	127

6.8.2	PDCCH multiplexing and scrambling	127

6.8.3	Modulation	128

6.8.4	Layer mapping and precoding	128

6.8.5	Mapping to resource elements	128

6.8A	Enhanced physical downlink control channel	129

6.8A.1	EPDCCH formats	130

6.8A.2	Scrambling	131

6.8A.3	Modulation	131

6.8A.4	Layer mapping and precoding	131

6.8A.5	Mapping to resource elements	132

6.8B	MTC physical downlink control channel	133

6.8B.1	MPDCCH formats	133

6.8B.2	Scrambling	134

6.8B.3	Modulation	134

6.8B.4	Layer mapping and precoding	134

6.8B.5	Mapping to resource elements	134

6.8C	Short physical downlink control channel (SPDCCH)	138

6.8C.1	SPDCCH formats	138

6.8C.2	Scrambling	140

6.8C.3	Modulation	140

6.8C.4	Layer mapping and precoding	140

6.8C.5	Mapping to resource elements	140

6.9	Physical hybrid ARQ indicator channel	142

6.9.1	Modulation	143

6.9.2	Resource group alignment, layer mapping and precoding	143

6.9.3	Mapping to resource elements	145

6.10	Reference signals	147

6.10.1	Cell-specific Reference Signal (CRS)	147

6.10.1.1	Sequence generation	147

6.10.1.2	Mapping to resource elements	148

6.10.2	MBSFN reference signals	150

6.10.2.1	Sequence generation	150

6.10.2.1.1	Sequence generation for 15 kHz and 7.5 kHz subcarrier spacing	150

6.10.2.1.2	Sequence generation for 1.25 kHz subcarrier spacing	151

6.10.2.1.3	Sequence generation for 2.5 kHz subcarrier spacing	151

6.10.2.1.4	Sequence generation for 0.37 kHz subcarrier spacing	151

6.10.2.2	Mapping to resource elements	151

6.10.2.2.1	Mapping to resource elements for 15 kHz and 7.5 kHz subcarrier spacing	151

6.10.2.2.2	Mapping to resource elements for 1.25 kHz	153

6.10.2.2.3	Mapping to resource elements for 2.5 kHz subcarrier spacing	153

6.10.2.2.4	Mapping to resource elements for 0.37 kHz subcarrier spacing	154

6.10.3	UE-specific reference signals associated with PDSCH	154

6.10.3.1	Sequence generation	155

6.10.3.2	Mapping to resource elements	156

6.10.3A	Demodulation reference signals associated with EPDCCH, MPDCCH, or SPDCCH	163

6.10.3A.1	Sequence generation	164

6.10.3A.2	Mapping to resource elements	165

6.10.4	Positioning reference signals	167

6.10.4.1	Sequence generation	167

6.10.4.2	Mapping to resource elements	167

6.10.4.3	Positioning reference signal subframe configuration	169

6.10.5	CSI reference signals	170

6.10.5.1	Sequence generation	171

6.10.5.2	Mapping to resource elements	171

6.10.5.3	CSI reference signal subframe configuration	180

6.11	Synchronization signals	180

6.11.1	Primary synchronization signal (PSS)	180

6.11.1.1	Sequence generation	180

6.11.1.2	Mapping to resource elements	181

6.11.2	Secondary synchronization signal (SSS)	181

6.11.2.1	Sequence generation	181

6.11.2.2	Mapping to resource elements	183

6.11.3	Resynchronization signal (RSS)	184

6.11.3.1	Sequence generation	184

6.11.3.2	Mapping to resource elements	184

6.11A	Discovery signal	185

6.11B	MTC wake-up signal (MWUS)	186

6.11B.1	Sequence generation	186

6.11B.2	Mapping to resource elements	186

6.12	OFDM baseband signal generation	187

6.13	Modulation and upconversion	187

7	Generic functions	189

7.1	Modulation mapper	189

7.1.1	BPSK	189

7.1.2	QPSK	189

7.1.3	16QAM	189

7.1.4	64QAM	190

7.1.5	256QAM	192

7.1.6	1024QAM	193

7.2	Pseudo-random sequence generation	193

8	Timing	193

8.1	Uplink-downlink frame timing	194

9	Sidelink	196

9.1	Overview	196

9.1.1	Physical channels	196

9.1.2	Physical signals	196

9.1.3	Handling of simultaneous sidelink and uplink/downlink transmissions	196

9.2	Slot structure and physical resources	197

9.2.1	Resource grid	197

9.2.2	Resource elements	197

9.2.3	Resource blocks	198

9.2.4	Resource pool	198

9.2.5	Guard period	198

9.3	Physical Sidelink Shared Channel	198

9.3.1	Scrambling	198

9.3.2	Modulation	199

9.3.3	Layer mapping	199

9.3.4	Transform precoding	199

9.3.5	Precoding	199

9.3.6	Mapping to physical resources	199

9.4	Physical Sidelink Control Channel	200

9.4.1	Scrambling	200

9.4.2	Modulation	200

9.4.3	Layer mapping	200

9.4.4	Transform precoding	200

9.4.5	Precoding	200

9.4.6	Mapping to physical resources	200

9.5	Physical Sidelink Discovery Channel	201

9.5.1	Scrambling	201

9.5.2	Modulation	201

9.5.3	Layer mapping	201

9.5.4	Transform precoding	201

9.5.5	Precoding	201

9.5.6	Mapping to physical resources	201

9.6	Physical Sidelink Broadcast Channel	202

9.6.1	Scrambling	202

9.6.2	Modulation	202

9.6.3	Layer mapping	202

9.6.4	Transform precoding	202

9.6.5	Precoding	202

9.6.6	Mapping to physical resources	202

9.7	Sidelink Synchronization Signals	202

9.7.1	Primary sidelink synchronization signal	203

9.7.1.1	Sequence generation	203

9.7.1.2	Mapping to resource elements	203

9.7.2	Secondary sidelink synchronization signal	203

9.7.2.1	Sequence generation	203

9.7.2.2	Mapping to resource elements	203

9.8	Demodulation reference signals	203

9.9	SC-FDMA baseband signal generation	205

9.10	Timing	205

10	Narrowband IoT	206

10.0	General	206

10.0.1	Frame structure	206

10.0.1.1	Frame structure type 1	206

10.0.1.2	Frame structure type 2	206

10.1	Uplink	207

10.1.1	Overview	207

10.1.1.1	Physical channels	207

10.1.1.2	Physical signals	207

10.1.2	Slot structure and physical resources	207

10.1.2.1	Resource grid	207

10.1.2.2	Resource elements	208

10.1.2.3	Resource unit	208

10.1.3	Narrowband physical uplink shared channel	209

10.1.3.1	Scrambling	209

10.1.3.2	Modulation	209

10.1.3.3	Layer mapping	210

10.1.3.4	Transform precoding	210

10.1.3.5	Precoding	210

10.1.3.6	Mapping to physical resources	210

10.1.4	Demodulation reference signal	212

10.1.4.1	Reference signal sequence	212

10.1.4.1.1	Reference signal sequence for 	212

10.1.4.1.2	Reference signal sequence for 	212

10.1.4.1.3	Group hopping	214

10.1.4.2	Mapping to physical resources	214

10.1.5	SC-FDMA baseband signal generation	215

10.1.6	Narrowband physical random-access channel	216

10.1.6.1	Time and frequency structure	216

10.1.6.2	Baseband signal generation	219

10.1.7	Modulation and upconversion	220

10.2	Downlink	220

10.2.1	Overview	220

10.2.1.1	Physical channels	220

10.2.1.2	Physical signals	220

10.2.2	Slot structure and physical resource elements	220

10.2.2.1	Resource grid	220

10.2.2.2	Resource elements	221

10.2.2.3	Guard period for half-duplex FDD operation	221

10.2.2.4	Guard period for TDD operation	221

10.2.3	Narrowband physical downlink shared channel	221

10.2.3.1	Scrambling	221

10.2.3.2	Modulation	221

10.2.3.3	Layer mapping and precoding	221

10.2.3.4	Mapping to resource elements	222

10.2.4	Narrowband physical broadcast channel	223

10.2.4.1	Scrambling	223

10.2.4.2	Modulation	223

10.2.4.3	Layer mapping and precoding	224

10.2.4.4	Mapping to resource elements	224

10.2.5	Narrowband physical downlink control channel	224

10.2.5.1	NPDCCH formats	224

10.2.5.2	Scrambling	225

10.2.5.3	Modulation	225

10.2.5.4	Layer mapping and precoding	225

10.2.5.5	Mapping to resource elements	225

10.2.6	Narrowband reference signal (NRS)	226

10.2.6.1	Sequence generation	229

10.2.6.2	Mapping to resource elements	229

10.2.6A	Narrowband positioning reference signal (NPRS)	231

10.2.6A.1	Sequence generation	231

10.2.6A.2	Mapping to resource elements	232

10.2.6A.3	NPRS subframe configuration	233

10.2.6B	Narrowband wake up signal (NWUS)	234

10.2.6B.1	Sequence generation	234

10.2.6B.2	Mapping to resource elements	234

10.2.7	Synchronization signals	235

10.2.7.1	Narrowband primary synchronization signal (NPSS)	235

10.2.7.1.1	Sequence generation	235

10.2.7.1.2	Mapping to resource elements	235

10.2.7.2	Narrowband secondary synchronization signal (NSSS)	236

10.2.7.2.1	Sequence generation	236

10.2.7.2.2	Mapping to resource elements	236

10.2.8	OFDM baseband signal generation	237

10.2.9	Modulation and upconversion	238

Annex A (informative):	Change history	239