| 3GPP TS 38.133 V16.9.0 (2021-09) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Requirements for support of radio resource management (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	42

1	Scope	44

2	References	44

3	Definitions, symbols and abbreviations	45

3.1	Definitions	45

3.2	Symbols	46

3.3	Abbreviations	47

3.4	Test tolerances	50

3.5	Frequency bands grouping	50

3.5.1	Introduction	50

3.5.2	NR operating bands in FR1	50

3.5.3	NR operating bands in FR2	51

3.6	Applicability of requirements in this specification version	51

3.6.1	RRC connected state requirements in DRX	52

3.6.2	Number of serving carriers	52

3.6.2.1	Number of serving carriers for SA	52

3.6.2.2	Number of serving carriers for EN-DC	52

3.6.2.3	Number of serving carriers for NE-DC	52

3.6.2.4	Number of serving carriers for NR-DC	53

3.6.3	Applicability for intra-band FR2	53

3.6.4	Applicability for FR2 UE power classes	53

3.6.5	Applicability for SDL bands	53

3.6.6	Applicability of requirements for NGEN-DC operation	53

3.6.7	Applicability of QCL	53

3.6.9	Applicability of requirements for scheduling availability	54

3.6.10	Applicability of requirements for measurement restrictions	54

4	SA: RRC_IDLE state mobility	54

4.1	Cell Selection	54

4.2	Cell Re-selection	54

4.2.1	Introduction	54

4.2.2	Requirements	55

4.2.2.1	UE measurement capability	55

4.2.2.2	Measurement and evaluation of serving cell	55

4.2.2.3	Measurements of intra-frequency NR cells	56

4.2.2.4	Measurements of inter-frequency NR cells	57

4.2.2.5	Measurements of inter-RAT E-UTRAN cells	59

4.2.2.6	Maximum interruption in paging reception	61

4.2.2.7	General requirements	61

4.2.2.8	Minimum requirement at transitions	61

4.2.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	62

4.2.2.9.1	Introduction	62

4.2.2.9.2	Measurements for UE fulfilling low mobility criterion	62

4.2.2.9.3	Measurements for UE fulfilling not-at-cell edge criterion	63

4.2.2.9.4	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	64

4.2.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	64

4.2.2.10.1	Introduction	64

4.2.2.10.2	Measurements for UE fulfilling low mobility criterion	64

4.2.2.10.3	Measurements for UE fulfilling not-at-cell edge criterion	65

4.2.2.10.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	66

4.2.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	66

4.2.2.11.2	Measurements for UE fulfilling low mobility criterion	67

4.2.2.11.3	Measurements for UE fulfilling with not-at-cell edge criterion	68

4.2.2.11.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	69

4.2A	Cell Re-selection when subject to CCA	69

4.2A.1	Introduction	69

4.2A.2	Requirements	70

4.2A.2.1	UE measurement capability	70

4.2A.2.2	Measurement and evaluation when subject to CCA on the serving cell	70

4.2A.2.3	Measurements of intra-frequency NR cells when subject to CCA on the serving cell and target cell	71

4.2A.2.4	Measurements of inter-frequency NR cells when subject to CCA on the target cell	72

4.2A.2.5	Measurements of inter-RAT E-UTRAN cells when subject to CCA on the serving cell	74

4.2A.2.6	Maximum interruption in paging reception when subject to CCA on the target cell	74

4.2A.2.7	General requirements	74

4.3	Minimization of Drive Tests (MDT)	75

4.3.1	Introduction	75

4.3.2	Measurement Requirements	75

4.3.3	Requirements for Relative Time Stamp Accuracy	75

4.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	76

4.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	76

4.4	Idle Mode CA/DC Measurements	76

4.4.1	Introduction	76

4.4.2	Measurement Requirements	76

4.4.2.1	Detected cell requirement during state transition and Idle mode	76

4.4.2.2	Measurements of inter-frequency CA/DC candidate cells	77

4.4.2.3	Measurements on serving cell	78

4.4.2.4	Measurements of E-UTRAN inter-RAT DC candidate cells	78

5	SA: RRC_INACTIVE state mobility	80

5.1	Cell Re-selection	80

5.1.1	Introduction	80

5.1.2	Requirements	80

5.1.2.1	UE measurement capability	80

5.1.2.2	Measurement and evaluation of serving cell	80

5.1.2.3	Measurements of intra-frequency NR cells	80

5.1.2.4	Measurements of inter-frequency NR cells	80

5.1.2.5	Measurements of inter-RAT E-UTRAN cells	80

5.1.2.6	Maximum interruption in paging reception	80

5.1.2.7	General requirements	80

5.1A	Cell Re-selection with CCA	80

5.1A.1	Introduction	80

5.1A.2	Requirements	81

5.1A.2.1	UE measurement capability	81

5.1A.2.2	Measurement and evaluation when CCA is used on the serving cell	81

5.1A.2.3	Measurements of intra-frequency NR cells when CCA is used on the serving cell and target cell	81

5.1A.2.4	Measurements of inter-frequency NR cells when CCA is used on the target cell	81

5.1A.2.5	Measurements of inter-RAT E-UTRAN cells when CCA is used on the serving cell	81

5.1A.2.6	Maximum interruption in paging reception when CCA is used on the target cell	81

5.1A.2.7	General requirements	81

5.2	Void	81

5.3	Minimization of Drive Tests (MDT)	81

5.3.1	Introduction	81

5.3.2	Measurement Requirements	82

5.3.3	Requirements for Relative Time Stamp Accuracy	82

5.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	82

5.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	82

5.3.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	82

5.4	Idle Mode CA/DC Measurements	82

5.4.1	Introduction	82

5.4.2	Measurement Requirements	82

5.4.2.1	Detected cell requirement during state transition and Idle mode	83

5.4.2.2	Measurements of inter-frequency CA/DC candidate cells	83

5.4.2.3	Measurements on serving cell	83

5.4.2.4	Measurements on E-UTRAN inter-RAT DC candidate cells	83

6	RRC_CONNECTED state mobility	83

6.1	Handover	83

6.1.1	NR Handover	83

6.1.1.1	Introduction	83

6.1.1.2	NR FR1 - NR FR1 Handover	83

6.1.1.2.1	Handover delay	83

6.1.1.2.2	Interruption time	83

6.1.1.3	NR FR2- NR FR1 Handover	84

6.1.1.3.1	Handover delay	84

6.1.1.3.2	Interruption time	84

6.1.1.4	NR FR2- NR FR2 Handover	85

6.1.1.4.1	Handover delay	85

6.1.1.4.2	Interruption time	85

6.1.1.5	NR FR1- NR FR2 Handover	86

6.1.1.5.1	Handover delay	86

6.1.1.5.2	Interruption time	86

6.1.2	NR Handover to other RATs	87

6.1.2.1	NR – E-UTRAN Handover	87

6.1.2.1.1	Introduction	87

6.1.2.1.2	Handover delay	87

6.1.2.1.3	Interruption time	87

6.1.2.2	NR – UTRAN Handover	88

6.1.2.2.1	Introduction	88

6.1.2.2.2	Handover delay	88

6.1.2.2.3	Interruption time	88

6.1.3	NR DAPS Handover	89

6.1.3.1	Introduction	89

6.1.3.2	NR FR1 - NR FR1 DAPS Handover	89

6.1.3.2.1	DAPS handover delay	89

6.1.3.2.2	Interruption time	90

6.1.3.3	NR FR2- NR FR1 DAPS Handover	92

6.1.3.3.1	DAPS handover delay	92

6.1.3.3.2	Interruption time	92

6.1.3.4	NR FR1- NR FR2 DAPS Handover	93

6.1.3.4.1	DAPS handover delay	93

6.1.3.4.2	Interruption time	93

6.1.4	NR Conditional Handover	94

6.1.4.1	Introduction	94

6.1.4.2	NR FR1 – NR FR1 conditional handover	94

6.1.4.3	NR FR2 – NR FR1 conditional handover	95

6.1.4.4	NR FR2 – NR FR2 conditional handover	96

6.1.4.4.1	Handover delay	96

6.1.4.4.2	Measurement time	96

6.1.4.4.3	Preparation time	96

6.1.4.4.4	Interruption time	96

6.1.4.5	NR FR1 – NR FR2 conditional handover	97

6.1A	Void	97

6.1A.1	Void	97

6.1A.1.1	Void	97

6.1A.1.2	Void	97

6.1A.1.2.1	Void	97

6.1A.1.2.2	Void	97

6.1B	Handover to target cell using CCA	97

6.1B.1	NR Handover	97

6.1B.1.1	Introduction	97

6.1B.1.2	NR FR1 - NR FR1 Handover	98

6.1B.1.2.1	Handover delay	98

6.1B.1.2.2	Interruption time	98

6.2	RRC Connection Mobility Control	99

6.2.1	SA: RRC Re-establishment	99

6.2.1.1	Introduction	99

6.2.1.2	Requirements	99

6.2.1.2.1	UE Re-establishment delay requirement	99

6.2.1A	RRC Re-establishment with CCA	100

6.2.1A.1	Introduction	100

6.2.1A.2	Requirements	101

6.2.1A.2.1	UE Re-establishment with CCA delay requirement	101

6.2.2	Random access	102

6.2.2.1	Introduction	102

6.2.2.2	Requirements for 4-step RA type	103

6.2.2.2.1	Contention based random access	103

6.2.2.2.2	Non-Contention based random access	104

6.2.2.2.3	UE behaviour when configured with supplementary UL	105

6.2.2.3	Requirements for 2-step RA type	105

6.2.2.3.1	Contention based random access	105

6.2.2.3.2	Non-Contention based random access	106

6.2.2.3.3	UE behaviour when configured with supplementary UL	107

6.2.2A	Random access when CCA is used on target frequency	107

6.2.2A.1	Introduction	107

6.2.2A.2	Requirements for 4-step RA type	107

6.2.2A.2.1	Contention based random access	107

6.2.2A.2.2	Non-Contention based random access	108

6.2.2A.3	Requirements for 2-step RA type	109

6.2.2A.3.1	Contention based random access	110

6.2.2A.3.2	Non-Contention based random access	111

6.2.3	SA: RRC Connection Release with Redirection	111

6.2.3.1	Introduction	111

6.2.3.2	Requirements	112

6.2.3.2.1	RRC connection release with redirection to NR	112

6.2.3.2.2	RRC connection release with redirection to E-UTRAN	113

6.2.3.2.3	RRC connection release with redirection to NR carrier subject to CCA	113

7	Timing	114

7.1	UE transmit timing	114

7.1.1	Introduction	114

7.1.2	Requirements	114

7.1.2.1	Gradual timing adjustment	116

7.1.2.2	Void	116

7.2	UE timer accuracy	116

7.2.1	Introduction	116

7.2.2	Requirements	116

7.3	Timing advance	117

7.3.1	Introduction	117

7.3.2	Requirements	117

7.3.2.1	Timing Advance adjustment delay	117

7.3.2.2	Timing Advance adjustment accuracy	117

7.4	Cell phase synchronization accuracy	117

7.4.1	Definition	117

7.4.2	Minimum requirements	117

7.5	Maximum Transmission Timing Difference	117

7.5.1	Introduction	117

7.5.2	Minimum Requirements for inter-band EN-DC	118

7.5.2.1	Minimum Requirements for inter-band synchronous EN-DC	118

7.5.3	Minimum Requirements for intra-band EN-DC	118

7.5.4	Minimum Requirements for NR Carrier Aggregation	119

7.5.5	Minimum Requirements for inter-band NE-DC	119

7.5.5.1	Minimum Requirements for inter-band synchronous NE-DC	120

7.5.6	Minimum Requirements for inter-band NR DC	120

7.6	Maximum Receive Timing Difference	120

7.6.1	Introduction	120

7.6.2	Minimum Requirements for inter-band EN-DC	121

7.6.2.1	Minimum Requirements for inter-band synchronous EN-DC	121

7.6.3	Minimum Requirements for intra-band EN-DC	121

7.6.4	Minimum Requirements for NR Carrier Aggregation	122

7.6.5	Minimum Requirements for inter-band NE-DC	122

7.6.5.1	Minimum Requirements for inter-band synchronous NE-DC	123

7.6.6	Minimum Requirements for inter-band NR DC	123

7.7	deriveSSB-IndexFromCell tolerance	124

7.7.1	Minimum requirements	124

7.8	Void	124

8	Signalling characteristics	124

8.1	Radio Link Monitoring	124

8.1.1	Introduction	124

8.1.2	Requirements for SSB based radio link monitoring	125

8.1.2.1	Introduction	125

8.1.2.2	Minimum requirement	126

8.1.2.3	Measurement restrictions for SSB based RLM	128

8.1.3	Requirements for CSI-RS based radio link monitoring	128

8.1.3.1	Introduction	128

8.1.3.2	Minimum requirement	129

8.1.3.3	Measurement restrictions for CSI-RS based RLM	131

8.1.4	Minimum requirement at transitions	132

8.1.5	Minimum requirement for UE turning off the transmitter	132

8.1.6	Minimum requirement for L1 indication	132

8.1.7	Scheduling availability of UE during radio link monitoring	133

8.1.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	133

8.1.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	133

8.1.7.3	Scheduling availability of UE performing radio link monitoring on FR2	133

8.1.7.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	134

8.1A	Radio Link Monitoring with CCA on Target Frequency	134

8.1A.1	Introduction	134

8.1A.2	Requirements for SSB Based Radio Link Monitoring	135

8.1A.2.1	Introduction	135

8.1A.2.2	Minimum Requirement	136

8.1A.3	Minimum requirement at transitions	137

8.1A.4	Minimum requirement for UE turning off the transmitter	137

8.1A.5	Minimum requirement for L1 indication	138

8.1A.6	Scheduling availability of UE during radio link monitoring	138

8.1A.6.1	Scheduling availability of UE performing radio link monitoring with the same subcarrier spacing as PDSCH/PDCCH	138

8.1A.6.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH	138

8.2	Interruption	138

8.2.1	EN-DC Interruption	138

8.2.1.1	Introduction	138

8.2.1.2	Requirements	139

8.2.1.2.1	Interruptions at transitions between active and non-active during DRX	139

8.2.1.2.2	Interruptions at transitions from non-DRX to DRX	139

8.2.1.2.3	Interruptions at SCell addition/release	140

8.2.1.2.4	Interruptions at SCell activation/deactivation	141

8.2.1.2.5	Interruptions during measurements on SCC	142

8.2.1.2.6	Interruptions at UL carrier RRC reconfiguration	143

8.2.1.2.7	Interruptions due to Active BWP switching Requirement	144

8.2.1.2.8	Interruptions at direct SCell activation and hibernation	145

8.2.1.2.9	Interruptions at SCell hibernation	145

8.2.1.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	145

8.2.1.2.11	Interruptions due to UE-specific CBW change	146

8.2.1.2.12	 Interruptions at NR SRS carrier based switching	146

8.2.1.2.13	 Interruptions at E-UTRA SRS carrier based switching	147

8.2.1.2.14	DL Interruptions at switching between two uplink carriers	148

8.2.1.2.15	Interruptions due to SCell dormancy	149

8.2.1.2.16	 Interruptions when identifying CGI of an NR cell with autonomous gaps	149

8.2.1.2.17	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	150

8.2.2	SA: Interruptions with Standalone NR Carrier Aggregation	150

8.2.2.1	Introduction	150

8.2.2.2	Requirements	151

8.2.2.2.1	Interruptions at SCell addition/release	151

8.2.2.2.2	Interruptions at SCell activation/deactivation	152

8.2.2.2.3	Interruptions during measurements on deactivated SCC	153

8.2.2.2.4	Interruptions at UL carrier RRC reconfiguration	154

8.2.2.2.5	Interruptions due to Active BWP switching Requirement	154

8.2.2.2.6	Interruptions at inter-frequency SFTD measurement	155

8.2.2.2.7	Interruptions at SCell activation/deactivation with multiple downlink SCells	156

8.2.2.2.8	Interruptions due to UE-specific CBW change	156

8.2.2.2.9	 Interruptions at NR SRS carrier based switching	156

8.2.2.2.10	DL Interruptions at UE switching between two uplink carriers	158

8.2.2.2.11	Interruptions at direct SCell activation	158

8.2.2.2.12	Interruptions due to SCell dormancy	159

8.2.2.2.13	Interruptions at transitions between active and non-active during DRX	159

8.2.2.2.14	 Interruptions when identifying CGI of an NR cell with autonomous gaps	159

8.2.2.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	160

8.2.3	NE-DC Interruptions	160

8.2.3.1	Introduction	160

8.2.3.2	Requirements	161

8.2.3.2.1	Interruptions at transitions between active and non-active during DRX	161

8.2.3.2.2	Interruptions at transitions from non-DRX to DRX	161

8.2.3.2.3	Interruptions at PSCell/SCell addition/release	161

8.2.3.2.4	Interruptions at SCell activation/deactivation	162

8.2.3.2.5	Interruptions during measurements on SCC	164

8.2.3.2.6	Interruptions at UL carrier RRC reconfiguration	165

8.2.3.2.7	Interruptions due to Active BWP switching Requirement	165

8.2.3.2.8	Interruptions at direct SCell activation and hibernation	165

8.2.3.2.9	Interruptions at SCell hibernation	166

8.2.3.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	166

8.2.3.2.11	 Interruptions at NR SRS carrier based switching	166

8.2.3.2.12	 Interruptions at E-UTRA SRS carrier based switching	168

8.2.3.2.13	Interruptions due to SCell dormancy	168

8.2.3.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	169

8.2.3.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	169

8.2.4	NR-DC: Interruptions	170

8.2.4.1	Introduction	170

8.2.4.2	Requirements	171

8.2.4.2.1	Interruptions at PSCell/SCell addition/release	171

8.2.4.2.2	Interruptions at SCell activation/deactivation	172

8.2.4.2.3	Interruptions during measurements on SCC	173

8.2.4.2.4	Interruptions at UL carrier RRC reconfiguration	173

8.2.4.2.5	Interruptions due to Active BWP switching Requirement	173

8.2.4.2.6	Interruptions at transitions between active and non-active during DRX	174

8.2.4.2.7	Interruptions at transitions from non-DRX to DRX	174

8.2.4.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	174

8.2.4.2.9	Interruptions at NR SRS carrier based switching	174

8.2.4.2.10	Interruptions at direct SCell activation	176

8.2.4.2.11	 Interruptions when identifying CGI of an NR cell with autonomous gaps	176

8.2.4.2.12	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	177

8.2.4.2.13	 Interruptions due to SCell dormancy	177

8.2.4.2A	Void	178

8.2.4.2A.1	Void	178

8.2.4.2A.2	Void	178

8.2.4.2A.3	Void	178

8.3	SCell Activation and Deactivation Delay	178

8.3.1	Introduction	178

8.3.2	SCell Activation Delay Requirement for Deactivated SCell	178

8.3.3	SCell Deactivation Delay Requirement for Activated SCell	182

8.3.4	Direct SCell Activation at SCell addition	183

8.3.5	Direct SCell Activation at Handover	184

8.3.7	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	186

8.3.8	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	189

8.3.9	Direct SCell Activation of Multiple Downlink SCells at SCell addition	189

8.3.10	Direct SCell Activation of Multiple Downlink SCells at Handover	191

8.3A	SCell Activation and Deactivation Delay in Carriers with CCA	192

8.3A.1	Introduction	192

8.3A.2	SCell Activation Delay Requirement for Deactivated SCell	192

8.3A.3	SCell Deactivation Delay Requirement for Activated SCell	195

8.4	UE UL carrier RRC reconfiguration delay	195

8.4.1	Introduction	195

8.4.2	UE UL carrier configuration delay requirement	195

8.4.3	UE UL carrier deconfiguration delay requirement	196

8.5	Link Recovery Procedures	196

8.5.1	Introduction	196

8.5.2	Requirements for SSB based beam failure detection	197

8.5.2.1	Introduction	197

8.5.2.2	Minimum requirement	197

8.5.2.3	Measurement restriction for SSB based beam failure detection	199

8.5.3	Requirements for CSI-RS based beam failure detection	199

8.5.3.1	Introduction	199

8.5.3.2	Minimum requirement	200

8.5.3.3	Measurement restrictions for CSI-RS beam failure detection	202

8.5.4	Minimum requirement for L1 indication	203

8.5.5	Requirements for SSB based candidate beam detection	203

8.5.5.1	Introduction	203

8.5.5.2	Minimum requirement	203

8.5.5.3	Measurement restriction for SSB based candidate beam detection	205

8.5.6	Requirements for CSI-RS based candidate beam detection	206

8.5.6.1	Introduction	206

8.5.6.2	Minimum requirement	206

8.5.6.3	Measurement restriction for CSI-RS based candidate beam detection	208

8.5.7	Scheduling availability of UE during beam failure detection	209

8.5.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	209

8.5.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	209

8.5.7.3	Scheduling availability of UE performing beam failure detection on FR2	209

8.5.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR DC	210

8.5.8	Scheduling availability of UE during candidate beam detection	210

8.5.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	210

8.5.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	210

8.5.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	210

8.5.8.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	211

8.5.9	Requirements for Beam Failure Recovery in SCell	211

8.5.9.1	Introduction	211

8.5.9.2	Requirement	211

8.5.10	Minimum requirement at transitions for beam failure detection	211

8.5A	Link Recovery Procedures when CCA is used on target frequency	212

8.5A.1	Introduction	212

8.5A.2	Requirements for SSB based beam failure detection	212

8.5A.2.1	Introduction	212

8.5A.2.2	Minimum requirement	213

8.5A.2.3	Measurement restriction for SSB based beam failure detection	213

8.5A.4	Minimum requirement for L1 indication	214

8.5A.5	Requirements for SSB based candidate beam detection	214

8.5A.5.1	Introduction	214

8.5A.5.2	Minimum requirement	214

8.5A.5.3	Measurement restriction for SSB based candidate beam detection	215

8.5A.7	Scheduling availability of UE during beam failure detection	215

8.5A.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH	215

8.5A.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH	215

8.5A.8	Scheduling availability of UE during candidate beam detection	215

8.5A.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH	215

8.5A.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH	216

8.6	Active BWP switch delay	216

8.6.1	Introduction	216

8.6.2	DCI and timer based BWP switch delay on a single CC	216

8.6.2A	DCI based BWP switch delay on multiple CCs	217

8.6.2A.1	Simultaneous DCI based BWP switch delay on multiple CCs	217

8.6.2A.2	Non-simultaneous DCI based BWP switch delay on multiple CCs	219

8.6.2B	Timer based BWP switch delay on multiple CCs	219

8.6.2B.1	Simultaneous timer based BWP switch delay on multiple CCs	219

8.6.2B.2	Non-simultaneous timer based BWP switch delay on multiple CCs	220

8.6.3	RRC based BWP switch delay on a single CC	220

8.6.3A	RRC based BWP switch delay on multiple CCs	221

8.6.3A.1	Simultaneous RRC based BWP switch delay on multiple CCs	221

8.6.3A.2	Non-simultaneous RRC based BWP switch delay on multiple CCs	221

8.6.4	BWP switch delay on Consistent UL CCA recovery	222

8.7	Void	222

8.8	NE-DC: E-UTRAN PSCell Addition and Release Delay	222

8.8.1	Introduction	222

8.8.2	E-UTRAN PSCell Addition Delay Requirement	222

8.8.3	E-UTRAN PSCell Release Delay Requirement	223

8.9	NR-DC: PSCell Addition and Release Delay	223

8.9.1	Introduction	223

8.9.2	PSCell Addition Delay Requirement	223

8.9.3	PSCell Release Delay Requirement	224

8.10	Active TCI state switching delay	224

8.10.4	DCI based TCI state switch delay	225

8.10.5	RRC based TCI state switch delay	225

8.10.6	Active TCI state list update delay	226

8.10A	Active TCI state switching delay with CCA	226

8.10A.1	Introduction	226

8.10A.2	Known conditions for TCI state	226

8.10A.3	MAC-CE based TCI state switch delay	227

8.10A.4	DCI based TCI state switch delay	227

8.10A.5	RRC based TCI state switch delay	227

8.10A.6	Active TCI state list update delay	228

8.11	PSCell Change	228

8.11A	void	229

8.11B	Conditional PSCell Change	229

8.11B.1	Introduction	229

8.11B.2	Conditoinal PSCell Change delay	229

8.11B.2.1	Measurement time	229

8.12	Uplink spatial relation switch delay	230

8.12.1	Introduction	230

8.12.2	Known conditions for spatial relation when associated with DL-RS	230

8.12.3	MAC-CE based spatial relation switch delay	230

8.12.4	DCI based spatial relation switch delay	231

8.12.5	RRC based spatial relation switch delay	231

8.13	UE-specific CBW change	231

8.13.1	Introduction	231

8.13.2	UE-specific CBW change delay	232

8.14	Pathloss reference signal switching delay	232

8.14.1	Introduction	232

8.14.2	Known conditions for pathloss reference signal	232

8.14.3	MAC-CE based pathloss reference signal switch delay	232

9	Measurement Procedure	233

9.1	General measurement requirement	233

9.1.1	Introduction	233

9.1.2	Measurement gap	233

9.1.2.1	EN-DC: Measurement Gap Sharing	245

9.1.2.1a	SA: Measurement Gap Sharing	245

9.1.2.1b	NE-DC: Measurement Gap Sharing	246

9.1.2.1c	NR-DC: Measurement Gap Sharing	247

9.1.3	UE Measurement capability	248

9.1.3.1	EN-DC: Monitoring of multiple layers using gaps	248

9.1.3.1a	SA: Monitoring of multiple layers using gaps	248

9.1.3.1b	NE-DC: Monitoring of multiple layers using gaps	249

9.1.3.1c	NR-DC: Monitoring of multiple layers using gaps	249

9.1.3.2	EN-DC: Maximum allowed layers for multiple monitoring	250

9.1.3.2a	SA: Maximum allowed layers for multiple monitoring	251

9.1.3.2b	NE-DC: Maximum allowed layers for multiple monitoring	251

9.1.3.2c	NR-DC: Maximum allowed layers for multiple monitoring	252

9.1A.3.2	Void	253

9.1.3A	UE Measurement capability under operation mode with CCA	253

9.1.3A.1	EN-DC: Monitoring of multiple layers using gaps under CCA	253

9.1.3A.1A	SA: Monitoring of multiple layers using gaps under CCA	253

9.1.3A.2	EN-DC: Maximum allowed layers for multiple monitoring under CCA	253

9.1A.3.2a	Void	254

9.1.3A.2A	SA: Maximum allowed layers for multiple monitoring under CCA	254

9.1.4	Capabilities for Support of Event Triggering and Reporting Criteria	254

9.1.4.1	Introduction	254

9.1.4.2	Requirements	254

9.1.5	Carrier-specific scaling factor	258

9.1.5.1	Monitoring of multiple layers outside gaps	258

9.1.5.1.1	EN-DC mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	260

9.1.5.1.2	SA mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	261

9.1.5.1.3	NR-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed outside gaps	262

9.1.5.1.4	NE-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based measurements performed outside gaps	263

9.1.5.2	Monitoring of multiple layers within gaps	264

9.1.5.2.1	EN-DC mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	265

9.1.5.2.2	SA mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	267

9.1.5.2.3	NE-DC: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed within gaps	268

9.1.5.2.4	NR-DC: carrier-specific scaling factor for SSB-based and CSI-RS-based L3 measurements performed within gaps	270

9.1.5.2.5	SA mode: carrier-specific scaling factor for PRS-based measurements performed within gaps	272

9.1.5.2.6	NE-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	272

9.1.5.2.7	NR-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	272

9.1.6	Minimum requirement at transitions	272

9.2	NR intra-frequency measurements	273

9.2.1	Introduction	273

9.2.2	Requirements applicability	273

9.2.3	Number of cells and number of SSB	274

9.2.3.1	Requirements for FR1	274

9.2.3.2	Requirements for FR2	274

9.2.4	Measurement Reporting Requirements	274

9.2.4.1	Periodic Reporting	274

9.2.4.2	Event-triggered Periodic Reporting	274

9.2.4.3	Event Triggered Reporting	275

9.2.5	Intrafrequency measurements without measurement gaps	275

9.2.5.1	Intrafrequency cell identification	275

9.2.5.2	Measurement period	278

9.2.5.3	Scheduling availability of UE during intra-frequency measurements	279

9.2.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	280

9.2.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	280

9.2.5.3.3	Scheduling availability of UE performing measurements on FR2	280

9.2.5.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	281

9.2.5.4	SFTD Measurements between PCell and PSCell	281

9.2.5.4.1	Introduction	281

9.2.5.4.2	SFTD Measurement delay	281

9.2.5.4.3	SFTD Measurement Reporting Delay	282

9.2.6	Intra-frequency measurements with measurement gaps	282

9.2.6.1	Void	282

9.2.6.2	Intra-frequency cell identification	282

9.2.6.3	Intrafrequency Measurement Period	284

9.2A	NR intra-frequency measurements with CCA	285

9.2A.1	Introduction	285

9.2A.2	Requirements applicability	286

9.2A.3	Number of cells and number of SSB	286

9.2A.4	Measurement Reporting Requirements	286

9.2A.5	Intra-frequency measurements without measurement gaps	287

9.2A.5.2	Measurement period	289

9.2A.5.3	Scheduling availability of UE during intra-frequency measurements	290

9.2A.5.3.1	Scheduling availability of UE performing measurements in TDD bands	290

9.2A.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH	291

9.2A.6	Intra-frequency measurements with measurement gaps	291

9.2A.6.1	Intra-frequency cell identification	291

9.2A.6.2	Intra-frequency Measurement Period	292

9.2A.7	Intra-frequency RSSI and Channel occupancy measurements	293

9.2A.7.1	Intra-frequency RSSI measurements	293

9.2A.7.2	Intra-frequency Channel occupancy measurements	294

9.2A.7.3	Scheduling restriction during RSSI and Channel Occupancy measurements	295

9.3	NR inter-frequency measurements	295

9.3.1	Introduction	295

9.3.2	Requirements applicability	296

9.3.2.1	Void	296

9.3.2.2	Void	296

9.3.3	Number of cells and number of SSB	296

9.3.3.1	Requirements for FR1	296

9.3.3.2	Requirements for FR2	296

9.3.4	Inter-frequency measurement with measurement gaps	296

9.3.4.1	Void	298

9.3.4.2	Void	298

9.3.5	Inter-frequency measurements	298

9.3.5.1	Void	298

9.3.5.2	Void	298

9.3.5.3	Void	298

9.3.6	Inter-frequency measurements reporting requirements	298

9.3.6.1	Periodic Reporting	298

9.3.6.2	Event-triggered Periodic Reporting	299

9.3.6.3	Event-triggered Reporting	299

9.3.7	Void	299

9.3.8	Inter-frequency SFTD measurement requirements	299

9.3.8.1	Introduction	299

9.3.8.2	SFTD Measurement delay	299

9.3.8.3	SFTD Measurement reporting delay	300

9.3.9	Inter frequency measurements without measurement gaps	301

9.3.9.1	Inter frequency Cell identification	301

9.3.9.2	Measurement period	302

9.3.9.3	Scheduling availability of UE during inter-frequency measurements	303

9.3.9.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	303

9.3.9.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	304

9.3.9.3.3	Scheduling availability of UE performing measurements on FR2	304

9.3.9.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	304

9.3A	NR inter-frequency measurements in carrier frequencies with CCA	305

9.3A.1	Introduction	305

9.3A.2	Requirements applicability	305

9.3A.3	Number of cells and number of SSB	305

9.3A.3.1	Requirements	305

9.3A.4	Inter-frequency cell identification	306

9.3A.5	Inter-frequency measurements	307

9.3A.6	NR Inter-frequency measurements reporting requirements	308

9.3A.6.1	Periodic Reporting	308

9.3A.6.2	Event-triggered Periodic Reporting	308

9.3A.6.3	Event-triggered Reporting	308

9.3A.8	Inter-frequency RSSI measurements	308

9.3A.9	Inter-frequency channel occupancy measurements	309

9.4	Inter-RAT measurements	309

9.4.1	Introduction	309

9.4.2	NR − E-UTRAN FDD measurements	311

9.4.2.1	Introduction	311

9.4.2.2	Requirements when no DRX is used	311

9.4.2.3	Requirements when DRX is used	312

9.4.2.4	Measurement reporting requirements	313

9.4.2.4.1	Periodic Reporting	313

9.4.2.4.2	Event-Triggered Periodic Reporting	313

9.4.2.4.3	Event-Triggered Reporting	313

9.4.3	NR − E-UTRAN TDD measurements	313

9.4.3.1	Introduction	313

9.4.3.2	Requirements when no DRX is used	313

9.4.3.3	Requirements when DRX is used	314

9.4.3.4	Measurement reporting requirements	316

9.4.3.4.1	Periodic Reporting	316

9.4.3.4.2	Event-Triggered Periodic Reporting	316

9.4.3.4.3	Event-Triggered Reporting	316

9.4.4	Inter-RAT RSTD measurements	316

9.4.4.1	NR − E-UTRAN FDD RSTD measurements	316

9.4.4.1.1	Introduction	316

9.4.4.1.2	Requirements	317

9.4.4.2	NR − E-UTRAN TDD RSTD measurements	320

9.4.4.2.1	Introduction	320

9.4.4.2.2	Requirements	321

9.4.5	Inter-RAT E-CID measurements	324

9.4.5.1	NR−E-UTRAN FDD E-CID RSRP and RSRQ measurements	324

9.4.5.1.1	Introduction	324

9.4.5.1.2	Requirements	324

9.4.5.1.3	Measurement Reporting Delay	324

9.4.5.2	NR−E-UTRAN TDD E-CID RSRP and RSRQ measurements	324

9.4.5.2.1	Introduction	324

9.4.5.2.2	Requirements	324

9.4.5.2.3	Measurement Reporting Delay	325

9.4.6	NR − UTRAN FDD measurements	325

9.4.6.1	Introduction	325

9.4.6.2	Requirements when no DRX is used	325

9.4.6.3	Requirements when DRX is used	326

9.4.7	NR – E-UTRAN measurements with autonomous gaps	328

9.4.7.1	CGI identification of an E-UTRA cell with autonomous gaps	328

9.4.7.2	CGI reporting delay	328

9.5	L1-RSRP measurements for Reporting	329

9.5.1	Introduction	329

9.5.2	Requirements applicability	329

9.5.3	Measurement Reporting Requirements	329

9.5.3.1	Periodic Reporting	330

9.5.3.2	Semi-Persistent Reporting	330

9.5.3.3	Aperiodic Reporting	330

9.5.4	L1-RSRP measurement requirements	330

9.5.4.1	SSB based L1-RSRP Reporting	330

9.5.4.2	CSI-RS based L1-RSRP Reporting	332

9.5.4A	Void	335

9.5.4A.1	Void	335

9.5.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	335

9.5.5.1	Measurement restriction for SSB based L1-RSRP	335

9.5.5.2	Measurement restriction for CSI-RS based L1-RSRP	335

9.5.6	Scheduling availability of UE during L1-RSRP measurement	336

9.5.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	336

9.5.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	336

9.5.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	337

9.5.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	337

9.5A	L1-RSRP measurements for Reporting under CCA	338

9.5A.1	Introduction	338

9.5A.2	Requirements applicability	338

9.5A.3	Measurement Reporting Requirements	338

9.5A.3.1	Periodic Reporting	338

9.5A.3.2	Semi-Persistent Reporting	339

9.5A.3.3	Aperiodic Reporting	339

9.5A.4	L1-RSRP measurement requirements	339

9.5A.4.1	SSB based L1-RSRP Reporting	339

9.5A.5	Measurement restriction for L1-RSRP measurement	340

9.5A.5.1	Measurement restriction for SSB based L1-RSRP	340

9.5A.6	Scheduling availability of UE during L1-RSRP measurement	340

9.5A.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH	340

9.5A.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH	340

9.5A.6.3	Scheduling availability of UE performing L1-RSRP measurement in case of FR1-FR2 inter-band CA	341

9.6	NE-DC: Measurements	341

9.6.1	Introduction	341

9.6.2	SFTD Measurements	341

9.6.2.1	Introduction	341

9.6.2.2	SFTD Measurement requirements	341

9.7	Cross Link Interference measurements	342

9.7.1	Introduction	342

9.7.2	SRS-RSRP measurements	342

9.7.2.1	Introduction	342

9.7.2.2	Requirements applicability	342

9.7.2.3	Measurement Reporting Requirements	343

9.7.2.3.1	Periodic Reporting	343

9.7.2.3.2	Event-triggered Periodic Reporting	343

9.7.2.3.3	Event Triggered Reporting	343

9.7.2.4	Measurement capability	343

9.7.2.5	SRS-RSRP measurement period	343

9.7.3	CLI-RSSI measurements	344

9.7.3.1	Introduction	344

9.7.3.2	Requirements applicability	344

9.7.3.3	Measurement Reporting Requirements	344

9.7.3.3.1	Periodic Reporting	344

9.7.3.3.2	Event-triggered Periodic Reporting	344

9.7.3.3.3	Event Triggered Reporting	344

9.7.3.4	Measurement capability	344

9.7.3.5	CLI-RSSI measurement period	345

9.7.4	Scheduling availability of UE during CLI measurements	345

9.7.4.1	Scheduling availability of UE performing measurement on FR1	345

9.7.4.2	Scheduling availability of UE performing measurement on FR2	345

9.8	L1-SINR measurements for Reporting	346

9.8.1	Introduction	346

9.8.2	Requirements applicability	346

9.8.3	Measurement Reporting Requirements	347

9.8.3.1	Periodic Reporting	347

9.8.3.2	Semi-Persistent Reporting	347

9.8.4	L1-SINR measurement requirements	348

9.8.4.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	348

9.8.4.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	351

9.8.5	Measurement restriction for L1-SINR measurement	353

9.8.5.1	Measurement restriction if SSB configured for L1-SINR Measurement	353

9.8.5.2	Measurement restriction if CSI-RS configured for L1-SINR measurement	354

9.8.5.3	Measurement restriction if CSI-IM configured for L1-SINR measurement	354

9.8.6	Scheduling availability of UE during L1-SINR measurement	355

9.8.6.1	Scheduling availability of UE performing L1-SINR measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	355

9.8.6.2	Scheduling availability of UE performing L1-SINR measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	355

9.8.6.4	Scheduling availability of UE performing L1-SINR measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	356

9.9	 NR measurements for positioning	356

9.9.1	Introduction	356

9.9.2	RSTD measurements	357

9.9.2.1	Introduction	357

9.9.2.2	Requirements Applicability	357

9.9.2.3	Measurement Capability	357

9.9.2.4	Measurement Reporting Requirements	357

9.9.2.4.1	Void	357

9.9.2.4.2	Void	357

9.9.2.4.3	Void	357

9.9.2.5	Measurements Period Requirements	357

9.9.2.6	Void	359

9.9.3	PRS-RSRP measurements	359

9.9.3.1	Introduction	359

9.9.3.2	Requirements applicability	359

9.9.3.3	Measurement Capability	360

9.9.3.4	Measurement Reporting Requirements	360

9.9.3.5	Measurement Period Requirements	360

9.9.4	UE Rx-Tx time difference measurements	362

9.9.4.1 Introduction	362

9.9.4.2 Requirements Applicability	362

9.9.4.3 Measurement Capability	362

9.9.4.4 Measurement Reporting Requirements	362

9.9.4.5 Measurement Period Requirements	362

9.9.5	NR E-CID measurements	364

9.9.5.1	Introduction	364

9.9.5.2	Measurement Requirements	365

9.9.5.2.1	Intra-frequency Measurement Requirements	365

9.9.5.2.2	Inter-frequency Measurement Requirements	365

9.9.5.2.3	Measurement Reporting Delay	365

9.10	CSI-RS based L3 measurements	366

9.10.1	Introduction	366

9.10.2	CSI-RS based intra-frequency measurements	366

9.10.2.1	Introduction	366

9.10.2.2	Requirements applicability	366

9.10.2.3	Number of cells and number of CSI-RS	367

9.10.2.3.1	Requirements for FR1	367

9.10.2.3.2	Requirements for FR2	368

9.10.2.4	Measurement Reporting Requirements	368

9.10.2.4.1	Periodic Reporting	368

9.10.2.4.2	Event-triggered Periodic Reporting	368

9.10.2.4.3	Event Triggered Reporting	368

9.10.2.5	Intra-frequency measurements without measurement gaps	369

9.10.2.6	Scheduling availability of UE during CSI-RS based intra-frequency measurements	370

9.10.2.6.1	Scheduling availability of UE performing CSI-RS based measurements in TDD bands	370

9.10.2.6.2	Scheduling availability of UE performing CSI-RS based measurements in FR2	370

9.10.3	CSI-RS based Inter-frequency measurements	370

9.10.3.1	Introduction	370

9.10.3.2	Requirements applicability	371

9.10.3.3	Number of cells and number of CSI-RS resources	371

9.10.3.3.1	Requirements for FR1	371

9.10.3.3.2	Requirements for FR2	371

9.10.3.4	Measurements reporting requirements	372

9.10.3.4.1	Periodic Reporting	372

9.10.3.4.2	Event-triggered Periodic Reporting	372

9.10.3.4.3	Event-triggered Reporting	372

9.10.3.5	Inter frequency measurements with measurement gaps	372

9.11	NR measurements with autonomous gaps	373

9.11.1	Introduction	373

9.11.2	CGI identification of an NR cell with autonomous gaps	374

9.11.3	CGI reporting delay	374

10	Measurement Performance requirements	375

10.1	NR measurements	375

10.1.1	Introduction	375

10.1.2	Intra-frequency RSRP accuracy requirements for FR1	375

10.1.2.1	Intra-frequency SS-RSRP accuracy requirements	375

10.1.2.1.1	Absolute SS-RSRP Accuracy	375

10.1.2.1.2	Relative SS-RSRP Accuracy	376

10.1.2.2	Void	377

10.1.2.3	Intra-frequency CSI-RSRP accuracy requirements	377

10.1.2.3.1	Absolute CSI-RSRP Accuracy	377

10.1.2.3.2	Relative CSI-RSRP Accuracy	378

10.1.2B	Intra-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	379

10.1.2B.1	Intra-frequency SS-RSRP accuracy requirements	379

10.1.2B.1.1	Absolute SS-RSRP Accuracy	379

10.1.3	Intra-frequency RSRP accuracy requirements for FR2	380

10.1.3.1	Intra-frequency SS-RSRP accuracy requirements	380

10.1.3.1.1	Absolute SS-RSRP Accuracy	380

10.1.3.1.2	Relative SS-RSRP Accuracy	381

10.1.3.2	Void	382

10.1.3.3	Intra-frequency CSI-RSRP accuracy requirements	382

10.1.3.3.1	Absolute CSI-RSRP Accuracy	382

10.1.3.3.2	Relative CSI-RSRP Accuracy	382

10.1.3B	Intra-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	383

10.1.3B.1	Intra-frequency SS-RSRP accuracy requirements	383

10.1.3B.1.1	Absolute SS-RSRP Accuracy	384

10.1.4	Inter-frequency RSRP accuracy requirements for FR1	384

10.1.4.1	Inter-frequency SS-RSRP accuracy requirements	384

10.1.4.1.1	Absolute Accuracy of SS-RSRP in FR1	384

10.1.4.1.2	Relative Accuracy of SS-RSRP in FR1	385

10.1.4.2	Void	386

10.1.4.3	Inter-frequency CSI-RSRP accuracy requirements	386

10.1.4.3.1	Absolute Accuracy of CSI-RSRP in FR1	386

10.1.4.3.2	Relative Accuracy of CS-RSRP in FR1	387

10.1.4B	Inter-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	388

10.1.4B.1	Inter-frequency SS-RSRP accuracy requirements	388

10.1.4B.1.1	Absolute Accuracy of SS-RSRP in FR1	388

10.1.5	Inter-frequency RSRP accuracy requirements for FR2	389

10.1.5.1	Inter-frequency SS-RSRP accuracy requirements	389

10.1.5.1.1	Absolute SS-RSRP Accuracy	389

10.1.5.1.2	Relative SS-RSRP Accuracy	390

10.1.5.2	Void	391

10.1.5.3	Inter-frequency CSI-RSRP accuracy requirements	391

10.1.5.3.1	Absolute CSI-RSRP Accuracy	391

10.1.5.3.2	Relative CSI-RSRP Accuracy	391

10.1.5B	Inter-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	392

10.1.6 RSRP Measurement Report Mapping	393

10.1.7	Intra-frequency RSRQ accuracy requirements for FR1	395

10.1.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	395

10.1.7.1.1	Absolute SS-RSRQ Accuracy in FR1	395

10.1.7.2	Intra-frequency CSI-RSRQ accuracy requirements	396

10.1.7.2.1	Absolute CSI-RSRQ Accuracy	396

10.1.7B	Intra-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	397

10.1.7B.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	397

10.1.7B.1.1	Absolute SS-RSRQ Accuracy in FR1	397

10.1.8	Intra-frequency RSRQ accuracy requirements for FR2	398

10.1.8.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	398

10.1.8.1.1	Absolute SS-RSRQ Accuracy in FR2	398

10.1.8.2	Intra-frequency CSI-RSRQ accuracy requirements	399

10.1.8.2.1	Absolute CSI-RSRQ Accuracy	399

10.1.8B	Intra-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	400

10.1.8B.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	400

10.1.8B.1.1	Absolute SS-RSRQ Accuracy in FR2	400

10.1.9	Inter-frequency RSRQ accuracy requirements for FR1	401

10.1.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	401

10.1.9.1.1	Absolute Accuracy of SS-RSRQ in FR1	401

10.1.9.1.2	Relative Accuracy of SS-RSRQ in FR1	402

10.1.9.2	Inter-frequency CSI-RSRQ accuracy requirements	403

10.1.9.2.1	Absolute CSI-RSRQ Accuracy	403

10.1.9.2.2	Relative CSI-RSRQ Accuracy	404

10.1.9B	Inter-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	405

10.1.9B.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	405

10.1.9B.1.1	Absolute Accuracy of SS-RSRQ in FR1	405

10.1.10	Inter-frequency RSRQ accuracy requirements for FR2	406

10.1.10.2	Inter-frequency CSI-RSRQ accuracy requirements	408

10.1.10.2.1	Absolute CSI-RSRQ Accuracy	408

10.1.10.2.2	Relative CSI-RSRQ Accuracy	408

10.1.10B	Inter-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	409

10.1.10B.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	409

10.1.10B.1.1	Absolute Accuracy of SS-RSRQ in FR2	409

10.1.11	RSRQ report mapping	410

10.1.12	Intra-frequency SINR accuracy requirements for FR1	411

10.1.12.2	Intra-frequency CSI-SINR accuracy requirements in FR1	411

10.1.12.2.1	Absolute CSI-SINR Accuracy in FR1	411

10.1.13	Intra-frequency SINR accuracy requirements for FR2	412

10.1.13.2	Intra-frequency CSI-SINR accuracy requirements in FR2	413

10.1.13.2.1	Absolute CSI-SINR Accuracy in FR2	413

10.1.14	Inter-frequency SINR accuracy requirements for FR1	414

10.1.14.2	Inter-frequency CSI-SINR accuracy requirements in FR1	416

10.1.14.2.1	Aboslute Accuracy of CSI-SINR in FR1	416

10.1.15	Inter-frequency SINR accuracy requirements for FR2	418

10.1.15.2	Inter-frequency CSI-SINR accuracy requirements in FR2	420

10.1.15.2.1	Aboslute Accuracy of CSI-SINR in FR2	420

10.1.15.2.2	Relative Accuracy of CSI-SINR in FR2	420

10.1.16	SINR report mapping	421

10.1.17	Power Headroom	422

10.1.18	PCMAX,c,f	423

10.1.19	L1-RSRP accuracy requirements for FR1	423

10.1.20	L1-RSRP accuracy requirements for FR2	427

10.1.21	SFTD accuracy requirements	430

10.1.22	CLI measurement accuracy requirements	434

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	438

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	438

10.1.23	RSTD Measurements	440

10.1.23.1	Introduction	440

10.1.23.2	Measurement Accuracy Requirements	441

10.1.23.3	Report mapping	444

10.1.23.3.1	 Absolute DL RSTD Measurement Reporting	444

10.1.23.3.2	 Differential Reporting for DL RSTD Measurement	446

10.1.23.3.3	Additional Path Report Mapping for DL RSTD	448

10.1.24	PRS-RSRP Measurements	450

10.1.24.1	Introduction	450

10.1.24.2	Measurement Accuracy Requirements	450

10.1.24.2.1 Absolute PRS RSRP accuracy	450

10.1.24.3	Report mapping	454

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	454

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	455

10.1.25	UE Rx-Tx Time Difference Measurements	457

10.1.25.1	Introduction	457

10.1.25.2	Measurement Accuracy Requirements	458

10.1.25.3	Report mapping	462

10.1.25.3.1	 Absolute UE Rx-Tx Measurement Report Mapping	462

10.1.25.3.2	 Differential UE Rx-Tx Measurement Report Mapping	464

10.1.25.3.3	 Additional Path Report Mapping for UE Rx-Tx Time Difference	466

10.1.26	FR2 P-MPR report	468

10.1.26.1	Report mapping	468

10.1.27	L1-SINR accuracy requirements for FR1	468

10.1.27.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured	468

10.1.27.1.1	Absolute Accuracy	468

10.1.27.1.2	Relative Accuracy	469

10.1.27.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured	470

10.1.27.2.1	Absolute Accuracy	470

10.1.27.2.2	Relative Accuracy	471

10.1.27.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated IMR configured	473

10.1.27.3.1	Absolute Accuracy	473

10.1.27.3.2	Relative Accuracy	475

10.1.28	L1-SINR accuracy requirements for FR2	476

10.1.28.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured	476

10.1.28.1.1	Absolute Accuracy	476

10.1.28.1.2	Relative Accuracy	477

10.1.28.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured	478

10.1.28.2.1	Absolute Accuracy	478

10.1.28.2.2	Relative Accuracy	479

10.1.28.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated IMR configured	480

10.1.28.3.1	Absolute Accuracy	480

10.1.28.3.2	Relative Accuracy	481

10.1.29	Intra-frequency RSRQ accuracy requirements under CCA	483

10.1.29.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	483

10.1.29.1.1	Absolute SS-RSRQ Accuracy	483

10.1.30	Inter-frequency RSRQ accuracy requirements under CCA	483

10.1.30.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	483

10.1.30.1.1	Aboslute Accuracy of SS-RSRQ	483

10.1.30.1.2	Relative Accuracy of SS-RSRQ	484

10.1.31	Intra-frequency SINR accuracy requirements under CCA	485

10.1.32	Inter-frequency SINR accuracy requirements under CCA	485

10.1.33	L1-RSRP accuracy requirements under CCA	487

10.1.34	RSSI measurements under CCA	488

10.1.34.1	Intra-frequency absolute RSSI measurement accuracy requirements in FR1	488

10.1.34.2	Inter-frequency absolute RSSI measurement accuracy requirements in FR1	488

10.1.34.3	RSSI measurement report mapping	489

10.1.35	Channel occupancy measurements under CCA	489

10.1.35.1	Intra-frequency channel occupancy measurement accuracy requirements in FR1	489

10.1.35.2	Inter-frequency channel occupancy measurement accuracy requirements in FR1	489

10.1.36	Intra-frequency RSRP accuracy requirements under CCA	490

10.1.36.1	Intra-frequency SS-RSRP accuracy requirements in FR1	490

10.1.36.1.1	Absolute SS-RSRP Accuracy	490

10.1.36.1.2	Relative SS-RSRP Accuracy	490

10.1.37	Inter-frequency RSRP accuracy requirements under CCA	491

10.1.37.1	Inter-frequency SS-RSRP accuracy requirements in FR1	491

10.1.37.1.1	Absolute Accuracy of SS-RSRP	491

10.1.37.1.2	Relative Accuracy of SS-RSRP	492

10.2	E-UTRAN measurements	492

10.2.1	Introduction	492

10.2.2	E-UTRAN RSRP measurements	493

10.2.3	E-UTRAN RSRQ measurements	493

10.2.4	E-UTRAN RSTD measurements	493

10.2.5	E-UTRAN RS-SINR measurements	493

10.2.6	E-UTRAN RSRP measurements for CA/DC Idle Mode Measurements	493

10.2.7	E-UTRAN RSRQ measurements for CA/DC Idle Mode Measurements	494

10.3	UTRAN FDD Measurements	494

10.3.1	UTRAN FDD CPICH RSCP	494

10.3.2	UTRAN FDD CPICH Ec/No	495

10.4	V2X measurements	495

10.4.1	Introduction	495

10.4.2	Intra-frequency PSBCH-RSRP accuracy requirements for FR1	496

10.4.2.1	PSBCH-RSRP Absolute Accuracy	496

10.4.2.2	PSBCH-RSRP Relative Accuracy	496

10.4.3	Intra-Frequency SL-RSSI Measurement Accuracy Requirements for FR1	497

10.4.3.1	Absolute SL-RSSI Accuracy	497

10.4.4	Intra-Frequency L1 SL-RSRP Measurement Accuracy Requirements for FR1	497

10.4.4.1	Absolute L1 SL-RSRP Accuracy	497

11	Void	498

12	V2X Requirements	498

12.1	Introduction	498

12.2	UE Transmit Timing	499

12.2.1	Introduction	499

12.2.2	GNSS as synchronization reference source	499

12.2.3	NR Cell as synchronization reference source	499

12.2.4	E-URTAN Cell as synchronization reference source	500

12.2.5	SyncRef UE as synchronization reference source	500

12.3	Initiation/Cease of SLSS Transmissions	500

12.3.1	Introduction	500

12.3.1.1	Initiation/Cease of SLSS transmissions with NR cell as synchronization reference source	500

12.3.1.2	Initiation/Cease of SLSS transmissions with EUTRAN cell as synchronization reference source	501

12.3.1.3	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	502

12.3.1.4	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	502

12.4	Selection / Reselection of V2X Synchronization Reference Source	503

12.5	L1 SL-RSRP measurements	504

12.5.1	Introduction	504

12.5.2	SL-RSRP measurements	504

12.6	Congestion Control measurements	504

12.7	Interruption	504

12.7.1	Interruptions to WAN due to V2X Sidelink Communication	504

12.7.2	V2X Sidelink Communication Dropping due to synchronization source change	505

12.7.3	Interruptions to WAN due to switching between E-UTRA V2X Sidelink and NR V2X Sidelink	507

12.8	Reliability of GNSS signal	507

12.9	Scheduling availability	507

12.9.1	Scheduling availability of UE switching between E-UTRA sidelink and NR sidelink	507

13	Measurement Performance Requirements for NR gNB	508

13.1	UL-RTOA	508

13.1.1	Report mapping	508

13.2	gNB Rx-Tx time difference	510

13.2.1	Report mapping	510

13.2.2	Measurement Accuracy Requirements	512

13.2.2.1	Introduction	512

13.2.2.2	Requirements	512

13.3	UL SRS RSRP measurement	513

13.3.1	Report mapping	513

13.3.2	Measurement accuracy requirements	514

13.3.2.1	Introduction	514

13.3.2.2	Requirements	514

13.4	AoA/ZoA	515

13.4.1	Report mapping	515

Annex A (normative): Test Cases	423

A.1	Purpose of annex	423

A.2	Requirement classification for statistical testing	423

A.2.1	Types of requirements in TS 38.133	423

A.2.1.1	Time and delay requirements on UE higher layer actions	423

A.2.1.2	Measurements of power levels, relative powers and time	423

A.2.1.3	Implementation requirements	424

A.2.1.4	Physical layer timing requirements	424

A.2.1.5	Requirements under CCA	424

A.3	RRM test configurations	425

A.3.1	Reference measurement channels	425

A.3.1.1	PDSCH	425

A.3.1.1.1	FDD	425

A.3.1.1.2	TDD	426

A.3.1A	Reference measurement channels under CCA	429

A.3.1A.1	PDSCH	429

A.3.1A.1.1	TDD	429

A.3.1A.2	CORESET for RMSI scheduling	430

A.3.1A.2.1	TDD	430

A.3.1A.3	CORESET for RMC scheduling	431

A.3.1A.3.1	TDD	431

A.3.1A.4	TDD UL/DL configuration	432

A.3.1A.5	RMC burst transmission model	432

A.3.1.2	CORESET for RMSI scheduling	433

A.3.1.2.1	FDD	433

A.3.1.2.2	TDD	434

A.3.1.3	CORESET for RMC scheduling	437

A.3.1.3.1	FDD	437

A.3.1.3.2	TDD	438

A.3.1.4	TDD UL/DL configuration	441

A.3.2	OFDMA channel noise generator (OCNG)	442

A.3.2.1	Generic OFDMA Channel Noise Generator (OCNG)	442

A.3.2.1.1	OCNG pattern 1: Generic OCNG pattern for all unused REs	442

A.3.2.1.2	OCNG pattern 2: Generic OCNG pattern for all unused REs for 2AoA setup	443

A.3.2.1.3	OCNG pattern 3: Generic OCNG pattern for unused REs in the same bandwidth as CORESET	443

A.3.2.1.4	OCNG pattern 4: Generic OCNG pattern for all unused REs outside SSB slot(s)	444

A.3.2.2	Void	445

A.3.3	Reference DRX configurations	445

A.3.3.1	DRX Configuration 1: DRX cycle = 40 ms and TAT = 500 ms	445

A.3.3.2	DRX Configuration 2: DRX cycle = 640 ms and TAT = 500 ms	445

A.3.3.3	DRX Configuration 3: DRX cycle = 40 ms and TAT = Infinity	445

A.3.3.4	DRX Configuration 4: DRX cycle = 160 ms and TAT = Infinity	446

A.3.3.5	DRX Configuration 5: DRX cycle = 320 ms and TAT = Infinity	446

A.3.3.6	DRX Configuration 6: DRX cycle = 320 ms and TAT = 500 ms	446

A.3.3.7	DRX Configuration 7: DRX cycle = 640 ms and TAT = Infinity	447

A.3.3.8	DRX Configuration 8: DRX cycle = 320 ms and TAT = Infinity	447

A.3.3.9	DRX Configuration 9: DRX cycle = 40 ms and TAT = 500 ms	447

A.3.3.10	DRX Configuration 10: DRX cycle = 640 ms and TAT = 500 ms	448

A.3.3.11	DRX Configuration 11: DRX cycle = 20 ms and TAT = Infinity	448

A.3.3.12	DRX Configuration 12: DRX cycle = 640 ms and TAT = Infinity	448

A.3.4	Test Cases with Different Channel Bandwidths	448

A.3.4.1	Test Cases with Different E-UTRA Channel Bandwidths	448

A.3.4.1.1	Introduction	448

A.3.4.1.2	Principle of testing	449

A.3.5	Test Cases for Synchronous and Asynchronous DC Operations	449

A.3.5.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations	449

A.3.5.1.1	Introduction	449

A.3.5.1.2	Principle of Testing	449

A.3.6	Antenna configurations	449

A.3.6.1	Antenna configurations for FR1	449

A.3.6.1.1	Antenna connection for 4 Rx capable UEs	449

A.3.6.1.1.1	Introduction	449

A.3.6.1.1.2	Principle of testing	449

A.3.6.2	Antenna configurations for FR2	452

A.3.6A	Antenna configurations with unlicensed bands	452

A.3.6A.1	Antenna configurations for FR1	452

A.3.6A.1.1	Antenna connection for 4 Rx capable UEs	452

A.3.6A.1.1.1	Introduction	452

A.3.6A.1.1.2	Principle of testing	452

A.3.7	EN-DC test setup	454

A.3.7.1	Introduction	454

A.3.7.2	E-UTRAN Serving Cell Parameters	454

A.3.7.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	454

A.3.7.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	455

A.3.7A	NR FR1-FR2 test setup	456

A.3.7B	EN-DC test setup with unlicensed bands	457

A.3.7B.1	Introduction	457

A.3.7B.2	E-UTRAN Serving Cell Parameters	457

A.3.7B.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) under CCA in FR1	457

A.3.7C	LTE-FR1/FR2 test setup	458

A.3.8	PRACH configurations	458

A.3.8.1	Introduction	458

A.3.8.2	PRACH configurations in FR1	458

A.3.8.2.1	FR1 PRACH configuration 1	458

A.3.8.2.2	FR1 PRACH configuration 2	459

A.3.8.2.3	FR1 PRACH configuration 3	460

A.3.8.2.4	FR1 PRACH configuration 4	461

A.3.8.3	PRACH configurations in FR2	462

A.3.8.3.1	FR2 PRACH configuration 1	462

A.3.8.3.2	FR2 PRACH configuration 2	463

A.3.8.3.3	FR2 PRACH configuration 3	464

A.3.8.3.4	FR2 PRACH configuration 4	465

A.3.8A	PRACH configurations under CCA	466

A.3.8A.1	Introduction	466

A.3.8A.2	PRACH configurations in FR1	466

A.3.8A.2.1	FR1 PRACH configuration 1 under CCA	466

A.3.8A.2.2	FR1 PRACH configuration 2 under CCA	467

A.3.9	BWP configurations	468

A.3.9.1	Introduction	468

A.3.9.2	Downlink BWP configurations	469

A.3.9.2.1	Initial BWP	469

A.3.9.2.2	Dedicated BWP	469

A.3.9.3	Uplink BWP configurations	469

A.3.9.3.1	Initial BWP	469

A.3.9.3.2	Dedicated BWP	470

A.3.10	SSB Configurations	470

A.3.10.1	SSB Configurations for FR1	470

A.3.10.1.1	SSB pattern 1 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	470

A.3.10.1.5	SSB pattern 5 in FR1: SSB allocation for SSB SCS=15 kHz starting from odd SFN in 10 MHz	472

A.3.10A	SSB Configurations under CCA	477

A.3.10A.1	SSB Configurations under CCA for FR1	477

A.3.10A.1.1	SSB pattern 1 under CCA for semi-static channel access: SSB allocation for SSB SCS=30kHz in 40MHz	477

A.3.10A.1.2	SSB pattern 2 under CCA for dynamic channel access: SSB allocation for SSB SCS=30kHz in 40MHz	478

A.3.10A.1.3	SSB pattern 3 under CCA for semi-static channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	478

A.3.10A.1.4	SSB pattern 4 under CCA for dynamic channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	479

A.3.11	SMTC Configurations	479

A.3.11.1	SMTC pattern 1: SMTC period = 20 ms with SMTC duration = 1 ms	479

A.3.11.2	SMTC pattern 2: SMTC period = 20 ms with SMTC duration = 5 ms	479

A.3.11.3	SMTC pattern 3: SMTC period = 160 ms with SMTC duration = 1 ms	480

A.3.11.4	SMTC pattern 4: SMTC period = 20 ms with SMTC duration = 1 ms	480

A.3.11.5	SMTC pattern 5: SMTC period = 20 ms with SMTC duration = 5 ms	480

A.3.11.6	SMTC pattern 6: SMTC period = 20 ms with SMTC duration = 5 ms	480

A.3.12	Test Cases with Different CC Configurations	480

A.3.12.1 EN-DC Test Cases with Different EN-DC Configurations	480

A.3.12.1.1	Introduction	480

A.3.12.1.2	Principle of testing	481

A.3.12.2	Carrier Aggregation Test Cases with Different CA Configurations	481

A.3.12.2.1	Introduction	481

A.3.12.2.2	Principle of testing	481

A.3.13	Test Cases in SA and EN-DC Operations	481

A.3.13.1	Introduction	481

A.3.13.2	Principle of Testing	481

A.3.13A Test Cases involving E-UTRA/FR1 and FR2 carriers	482

A.3.13A.1	Introduction	482

A.3.13A.2	Principle of Testing in EN-DC	482

A.3.13A.3	Principle of Testing in SA	482

A.3.14	CSI-RS configurations	483

A.3.14.1	FDD	483

A.3.14.2	TDD	485

A.3.15	Angle of Arrival (AoA) for FR2 RRM test cases	492

A.3.15.1	Setup 1: Single AoA in Rx beam peak direction	492

A.3.15.2	Setup 2: Single AoA in non Rx beam peak direction	492

A.3.15.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction	492

A.3.15.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction	493

A.3.15.3	Setup 3: 2 AoAs	493

A.3.15.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak	493

A.3.15.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction	493

A.3.15.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction	493

A.3.16	TCI State Configuration	494

A.3.16.1	Introduction	494

A.3.16.2	TCI states	494

A.3.17	Configurations of CSI-RS for tracking	494

A.3.17.1	Configuration of CSI-RS for tracking for FR1	494

A.3.17.1.1	FDD	494

A.3.17.1.2	TDD	495

A.3.17.2	Configuration of CSI-RS for tracking for FR2	496

A.3.17.2.1	TDD	496

A.3.18	Additional definitions related to OTA testing for FR2 RRM test cases	497

A.3.18.1	Introduction	497

A.3.18.2	PRACH Power Measurement	497

A.3.19	Test applicability for DAPS handover	497

A.3.19.1	Introduction	497

A.3.19.2	Principle of testing	497

A.3.20	MsgA configurations	498

A.3.20.1	Introduction	498

A.3.20.2	MsgA configurations in FR1	498

A.3.20.2.1	FR1 MsgA configuration 1	498

A.3.20.2.2	FR1 MsgA configuration 2	499

A.3.20.3	MsgA configurations in FR2	501

A.3.20.3.1	FR2 MsgA configuration 1	501

A.3.20.3.2	FR2 MsgA configuration 2	502

A.3.20A	MsgA configurations under CCA	504

A.3.20A.1	Introduction	504

A.3.20A.2	MsgA configurations in FR1	504

A.3.20A.2.1	FR1 MsgA configuration 1 under CCA	504

A.3.20A.2.2	FR1 MsgA configuration 2 under CCA	505

A.3.21	V2X sidelink communication	508

A.3.21.1	Introduction	508

A.3.21.2	Reference resource pool configurations for V2X Sidelink Communication	508

A.3.21.3	Reference measurement channels for V2X Sidelink Communication	515

A.3.22	CSI-IM configurations	515

A.3.22.1	FDD	515

A.3.22.2	TDD	516

A.3.23	Spatial Relation Configuration	518

A.3.23.1	Introduction	518

A.3.23.2	Spatial Relation	518

A.3.24	SRS configuration	519

A.3.25	Channel bandwidth (CBW) configurations	521

A.3.25.1	DL UE specific CBW	521

A.3.25.2	UL UE specific CBW	521

A.3.26	CCA model	521

A.3.26.1	Introduction	521

A.3.26.2	CCA model for operation on a carrier frequency with CCA in FR1	521

A.3.26.2.1	DL CCA model	521

A.3.26.2.2	UL CCA model	522

A.3.27	Test Cases with at Least One Cell on a Carrier Frequency with CCA	523

A.3.27.1	Introduction	523

A.3.27.2	 NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	523

A.3.27.3	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	523

A.3.27.4	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	523

A.3.27.5	E-UTRA Standalone Tests with at Least One NR Cell under CCA	524

A.3.28	Discovery Burst Transmission Window configuration under CCA	524

A.3.28.1	DBT Window pattern 1: DBT Window period = 20 ms with DBT Window duration = 1 ms	524

A.3.29	Testing principles for UE capable of only NR bands with shared spectrum access	524

A.3.29.1	Introduction	524

A.3.29.2	Principle of testing for UE capable of EN-DC with only NR bands with shared spectrum access	524

A.3.29.3	Principle of testing for UE capable of SA operation with only NR bands with shared spectrum access	525

A.3.30	CSI-RS configurations for RRM	525

A.3.30.1	FDD	525

A.3.30.2	TDD	526

A.3.31	PRS Configurations	528

A.3.31.1.	PRS Configurations for FR1	528

A.3.31.1.1.	PRS pattern 1 in FR1: SCS=15 KHz	528

A.3.31.1.2.	PRS pattern 2 in FR1: SCS=30 KHz	529

A.3.31.2.	PRS Configurations for FR2	529

A.3.31.2.1.	PRS pattern 1 in FR2: SCS=120 KHz	529

A.4	EN-DC tests with all NR cells in FR1	530

A.4.1	Void	530

A.4.2	Void	530

A.4.3	RRC_CONNECTED state mobility	530

A.4.3.1	Void	530

A.4.3.2	RRC Connection Mobility Control	530

A.4.3.2.1	Void	530

A.4.3.2.2	Random Access	530

A.4.3.2.2.1	4-step RA type contention based random access test in FR1 for PSCell in EN-DC	530

A.4.3.2.2.2	4-step RA type non-contention based random access test in FR1 for PSCell in EN-DC	534

A.4.3.2.2.3	2-step RA type contention based random access test in FR1 for PSCell in EN-DC	538

A.4.3.2.2.4	2-step RA type non-contention based random access test in FR1 for PSCell in EN-DC	542

A.4.3.2.3	Void	546

A.4.4	Timing	546

A.4.4.1	UE transmit timing	546

A.4.4.1.1	NR UE Transmit Timing Test for FR1	546

A.4.4.1.1.1	Test Purpose and environment	546

A.4.4.1.1.2	Test requirements	550

A.4.4.2	UE timer accuracy	550

A.4.4.3	Timing advance	550

A.4.4.3.1	EN-DC FR1 timing advance adjustment accuracy	550

A.4.4.3.1.1	Test Purpose and Environment	550

A.4.4.3.1.2	Test Parameters	550

A.4.4.3.1.3	Test Requirements	554

A.4.5	Signaling characteristics	554

A.4.5.1	Radio link Monitoring	554

A.4.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	555

A.4.5.1.1.1	Test Purpose and Environment	555

A.4.5.1.1.2	Test Requirements	559

A.4.5.1.2	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	559

A.4.5.1.2.1	Test Purpose and Environment	559

A.4.5.1.2.2	Test Requirements	565

A.4.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	565

A.4.5.1.3.1	Test Purpose and Environment	565

A.4.5.1.3.2	Test Requirements	570

A.4.5.1.4	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	570

A.4.5.1.4.1	Test Purpose and Environment	570

A.4.5.1.4.2	Test Requirements	576

A.4.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	576

A.4.5.1.5.1	Test Purpose and Environment	576

A.4.5.1.5.2	Test Requirements	581

A.4.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	581

A.4.5.1.6.1	Test Purpose and Environment	581

A.4.5.1.6.2	Test Requirements	587

A.4.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	587

A.4.5.1.7.1	Test Purpose and Environment	587

A.4.5.1.7.2	Test Requirements	592

A.4.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	593

A.4.5.1.8.1	Test Purpose and Environment	593

A.4.5.1.8.2	Test Requirements	598

A.4.5.2	Interruption	598

A.4.5.2.1	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	598

A.4.5.2.1.1	Test Purpose and Environment	598

A.4.5.2.1.2	Test Requirements	602

A.4.5.2.2	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	602

A.4.5.2.2.1	Test Purpose and Environment	602

A.4.5.2.2.2	Test Requirements	606

A.4.5.2.3	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	606

A.4.5.2.3.1	Test Purpose and Environment	606

A.4.5.2.3.2	Test Requirements	610

A.4.5.2.4	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	611

A.4.5.2.4.1	Test Purpose and Environment	611

A.4.5.2.4.2	Test Requirements	615

A.4.5.2.5	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	616

A.4.5.2.5.1	Test Purpose and Environment	616

A.4.5.2.5.2	Test Requirements	619

A.4.5.2.6	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	619

A.4.5.2.6.1	Test Purpose and Environment	619

A.4.5.2.6.2	Test Requirements	623

A.4.5.2.7	Void	623

A.4.5.2.8	E-UTRAN - NR FR1 interruptions at NR SRS carrier based switching in asynchronous EN-DC	623

A.4.5.2.8.1	Test Purpose and Environment	623

A.4.5.2.8.2	Test Requirements	626

A.4.5.2.9	E-UTRAN – NR interruptions at E-UTRA SRS carrier based switching	627

A.4.5.2.9.1	Test Purpose and Environment	627

A.4.5.2.9.2	Test Requirements	631

A.4.5.3	SCell Activation and Deactivation Delay	632

A.4.5.3.1	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	632

A.4.5.3.1.1	Test Purpose and Environment	632

A.4.5.3.1.2	Test Requirements	637

A.4.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 320 ms SCell measurement cycle	638

A.4.5.3.2.1	Test Purpose and Environment	638

A.4.5.3.2.2	Test Requirements	638

A.4.5.3.3	SCell Activation and deactivation of unknown SCell in FR1	638

A.4.5.3.3.1	Test Purpose and Environment	638

A.4.5.3.3.2	Test Requirements	639

A.4.5.3.4	SCell Activation and deactivation of multiple unknown SCells in FR1 with single activation/deactivation command	639

A.4.5.3.4.1	Test Purpose and Environment	639

A.4.5.3.4.2	Test Requirements	644

A.4.5.3.5	Direct SCell activation at SCell addition of known SCell in FR1	644

A.4.5.3.5.1	Test Purpose and Environment	644

A.4.5.3.5.2	Test Requirements	648

A.4.5.4	UE UL carrier RRC reconfiguration Delay	648

A.4.5.4.1	UE UL carrier RRC reconfiguration Delay	648

A.4.5.4.1.1 Test Purpose and Environment	648

A.4.5.4.1.2	Test Requirements	656

A.4.5.5	Beam Failure Detection and Link recovery procedures	657

A.4.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	657

A.4.5.5.1.1	Test Purpose and Environment	657

A.4.5.5.1.2	Test Requirements	662

A.4.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	662

A.4.5.5.2.1	Test Purpose and Environment	662

A.4.5.5.2.2	Test Requirements	668

A.4.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	668

A.4.5.5.3.1	Test Purpose and Environment	668

A.4.5.5.3.2	Test Requirements	674

A.4.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in DRX mode	674

A.4.5.5.4.1	Test Purpose and Environment	674

A.4.5.5.4.2	Test Requirements	680

A.4.5.5.5	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	680

A.4.5.5.5.1	Test Purpose and Environment	680

A.4.5.5.5.2	Test Requirements	686

A.4.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	686

A.4.5.5.6.1	Test Purpose and Environment	686

A.4.5.5.6.2	Test Requirements	693

A.4.5.6	Active BWP switch	694

A.4.5.6.1	DCI-based and Timer-based Active BWP Switch	694

A.4.5.6.1.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	694

A.4.5.6.1.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	699

A.4.5.6.2	RRC-based Active BWP Switch	705

A.4.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	709

A.4.5.6.3.1	Simultaneous E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in EN-DC on multiple CCs	709

A.4.5.6.3.1.1	Test Purpose and Environment	709

A.4.5.6.3.1.2	Test Requirements	714

A.4.5.6.4	SCell dormancy switch	715

A.4.5.6.4.1	E-UTRAN – NR FR1 PSCell SCell dormancy switch of single FR1 SCell outside active time	715

A.4.5.6.4.1.1	Test Purpose and Environment	715

A.4.5.6.4.1.2	Test Requirements	720

A.4.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR1 SCells inside active time	720

A.4.5.6.4.2.1	Test Purpose and Environment	720

A.4.5.6.4.2.2	Test Requirements	725

A.4.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	725

A.4.5.6.5.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC on multiple CCs	725

A.4.5.6.5.1.1	Test Purpose and Environment	725

A.4.5.6.5.1.2	Test Requirements	730

A.4.5.7	PSCell addition and release delay	730

A.4.5.7.1	Addition and Release Delay of known NR PSCell	730

A.4.5.7.1.1	Test purpose and environment	730

A.4.5.7.1.2	Test Requirements	734

A.4.5.8	DL Interruptions at switching between two uplink carriers	735

A.4.5.8.1	Test Purpose and Environment	735

A.4.5.8.2	Test Requirements	738

A.4.5.9	UE specific CBW change	738

A.4.5.9.1	UE specific CBW change on FR1 NR PSCell with non-DRX in synchronous EN- DC	738

A.4.5.9.1.1	Test Purpose and Environment	738

A.4.5.9.1.2	Test Requirements	742

A.4.6	Measurement procedure	742

A.4.6.1	Intra-frequency Measurements	742

A.4.6.1.1	EN-DC event triggered reporting tests without gap under non-DRX	742

A.4.6.1.1.1	Test purpose and Environment	742

A.4.6.1.1.2	Test parameters	742

A.4.6.1.1.3	Test Requirements	746

A.4.6.1.2	EN-DC event triggered reporting tests without gap under DRX	746

A.4.6.1.2.1	Test purpose and Environment	746

A.4.6.1.2.2	Test parameters	746

A.4.6.1.2.2	Test Requirements	750

A.4.6.1.3	EN-DC event triggered reporting tests with per-UE gaps under non-DRX	750

A.4.6.1.3.1	Test purpose and Environment	750

A.4.6.1.3.2	Test parameters	750

A.4.6.1.3.3	Test Requirements	754

A.4.6.1.4	EN-DC event triggered reporting tests with per-UE gaps under DRX	754

A.4.6.1.4.1	Test purpose and Environment	754

A.4.6.1.4.2	Test parameters	754

A.4.6.1.4.3	Test Requirements	758

A.4.6.1.5	EN-DC event triggered reporting tests without gap under non-DRX with SSB index reading	758

A.4.6.1.5.1	Test purpose and Environment	758

A.4.6.1.5.2	Test parameters	758

A.4.6.1.5.3	Test Requirements	760

A.4.6.1.6	EN-DC event triggered reporting tests with SSB index reading with per-UE gaps	761

A.4.6.1.6.1	Test purpose and Environment	761

A.4.6.1.6.2	Test parameters	761

A.4.6.1.6.3	Test Requirements	763

A.4.6.1.7	EN-DC event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	764

A.4.6.1.7.1	Test purpose and Environment	764

A.4.6.1.7.2	Test parameters	764

A.4.6.1.7.3	Test Requirements	768

A.4.6.2	Inter-frequency Measurements	768

A.4.6.2.1	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	768

A.4.6.2.1.1	Test Purpose and Environment	768

A.4.6.2.1.2	Test Requirements	772

A.4.6.2.2	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	772

A.4.6.2.2.1	Test Purpose and Environment	772

A.4.6.2.2.2	Test Requirements	776

A.4.6.2.3	Void	777

A.4.6.2.4	Void	777

A.4.6.2.5	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	777

A.4.6.2.5.1	Test Purpose and Environment	777

A.4.6.2.5.2	Test Requirements	781

A.4.6.2.6	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	782

A.4.6.2.6.1	Test Purpose and Environment	782

A.4.6.2.6.2	Test Requirements	786

A.4.6.2.7	Void	787

A.4.6.2.8	Void	787

A.4.6.3	Void	787

A.4.6.4	L1-RSRP measurement for beam reporting	787

A.4.6.4.1	SSB based L1-RSRP measurement when DRX is not used	787

A.4.6.4.1.1	Test Purpose and Environment	787

A.4.6.4.1.2	Test parameters	787

A.4.6.4.1.3	Test Requirements	790

A.4.6.4.2	SSB based L1-RSRP measurement when DRX is used	790

A.4.6.4.2.1	Test Purpose and Environment	790

A.4.6.4.2.2	Test parameters	791

A.4.6.4.2.3	Test Requirements	794

A.4.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	794

A.4.6.4.3.1	Test Purpose and Environment	794

A.4.6.4.3.2	Test parameters	795

A.4.6.4.3.3	Test Requirements	798

A.4.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	798

A.4.6.4.4.1	Test Purpose and Environment	798

A.4.6.4.4.2	Test parameters	799

A.4.6.4.4.3	Test Requirements	801

A.4.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	801

A.4.6.4.5.1	Test Purpose and Environment	801

A.4.6.4.5.2	Test parameters	802

A.4.6.4.2.3	Test Requirements	805

A.4.6.5	CLI measurements	805

A.4.6.5.1	SRS-RSRP measurement with non-DRX	805

A.4.6.5.1.1	Test Purpose and Environment	805

A.4.6.5.1.2	Test Parameters	806

A.4.6.5.1.3	Test Requirements	809

A.4.6.5.2	CLI-RSSI measurement with non-DRX	809

A.4.6.5.2.1	Test Purpose and Environment	809

A.4.6.5.2.2	Test Parameters	810

A.4.6.5.2.3	Test Requirements	811

A.4.6.6	Measurements with autonomous gaps	812

A.4.6.6.1	EN-DC intra-frequency CGI identification of NR FR1 cell with autonomous gaps in synchronous EN-DC	812

A.4.6.6.1.1	Test Purpose and Environment	812

A.4.6.6.1.2	Test Requirements	816

A.4.6.7	L1-SINR measurement for beam reporting	816

A.4.6.7.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR when DRX is not used	816

A.4.6.7.1.1	Test Purpose and Environment	816

A.4.6.7.1.2	Test parameters	817

A.4.6.7.1.3	Test Requirements	820

A.4.6.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	820

A.4.6.7.2.1	Test Purpose and Environment	820

A.4.6.7.2.2	Test parameters	821

A.4.6.7.2.3	Test Requirements	824

A.4.6.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	824

A.4.6.7.3.1	Test Purpose and Environment	824

A.4.6.7.3.2	Test parameters	825

A.4.6.7.3.3	Test Requirements	828

A.4.6.8	CSI-RS based intra-frequency Measurement	828

A.4.6.8.1	EN-DC event triggered reporting tests without gap under DRX	828

A.4.6.8.1.1	Test purpose and Environment	828

A.4.6.8.1.2	Test Requirements	831

A.4.6.9	CSI-RS based inter-frequency Measurement	832

A.4.6.9.1	EN-DC event triggered reporting tests for FR1 cell when non-DRX is used	832

A.4.6.9.1.1	Test Purpose and Environment	832

A.4.6.9.1.2	Test Requirements	835

A.4.7	Measurement Performance requirements	836

A.4.7.1	SS-RSRP	836

A.4.7.1.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	836

A.4.7.1.1.1	Test Purpose and Environment	836

A.4.7.1.1.2	Test parameters	836

A.4.7.1.1.3	Test Requirements	840

A.4.7.1.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	840

A.4.7.1.2.1	Test Purpose and Environment	840

A.4.7.1.2.2	Test parameters	841

A.4.7.1.2.3	Test Requirements	845

A.4.7.1.3	Void	845

A.4.7.2	SS-RSRQ	845

A.4.7.2.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	845

A.4.7.2.1.1	Test Purpose and Environment	845

A.4.7.2.1.2	Test Parameters	846

A.4.7.2.1.3	Test Requirements	851

A.4.7.2.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	851

A.4.7.2.2.1	Test Purpose and Environment	851

A.4.7.2.2.2	Test Parameters	851

A.4.7.2.2.3	Test Requirements	856

A.4.7.3	SS-SINR	856

A.4.7.3.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	856

A.4.7.3.1.1	Test Purpose and Environment	856

A.4.7.3.1.2	Test Parameters	856

A.4.7.3.1.3	Test Requirements	860

A.4.7.3.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	860

A.4.7.3.2.1	Test Purpose and Environment	860

A.4.7.3.2.2	Test Parameters	860

A.4.7.3.2.3	Test Requirements	865

A.4.7.4	L1-RSRP measurement for beam reporting	865

A.4.7.4.1	SSB based L1-RSRP measurement	865

A.4.7.4.1.1	Test Purpose and Environment	865

A.4.7.4.1.2	Test parameters	866

A.4.7.4.1.3	Test Requirements	869

A.4.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	869

A.4.7.4.2.1	Test Purpose and Environment	869

A.4.7.4.2.2	Test parameters	870

A.4.7.4.2.3	Test Requirements	873

A.4.7.5	SFTD accuracy	873

A.4.7.5.1	SFTD accuracy	873

A.4.7.5.1.1	Test Purpose and Environment	873

A.4.7.5.1.2	Test Parameters	873

A.4.7.5.1.3	Test Requirements	878

A.4.7.5.2	Void	878

A.4.7.5.3	Void	878

A.4.7.6	CLI measurements	878

A.4.7.6.1	EN-DC SRS-RSRP measurement accuracy with FR1 serving cell	878

A.4.7.6.1.1	Test Purpose and Environment	878

A.4.7.6.1.2	Test parameters	879

A.4.7.6.1.3	Test Requirements	884

A.4.7.6.2	EN-DC CLI-RSSI measurement accuracy with FR1 serving cell	884

A.4.7.6.2.1	Test Purpose and Environment	884

A.4.7.6.2.2	Test parameters	885

A.4.7.6.2.3	Test Requirements	888

A.4.7.7	L1-SINR measurement for beam reporting	888

A.4.7.7.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	888

A.4.7.7.1.1	Test Purpose and Environment	888

A.4.7.7.1.2	Test parameters	889

A.4.7.7.1.3	Test Requirements	893

A.4.7.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR	893

A.4.7.7.2.1	Test Purpose and Environment	893

A.4.7.7.2.2	Test parameters	894

A.4.7.7.2.3	Test Requirements	898

A.4.7.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	898

A.4.7.7.3.1	Test Purpose and Environment	898

A.4.7.7.3.2	Test parameters	899

A.4.7.7.3.3	Test Requirements	902

A.4.7.8	CSI-RSRP	902

A.4.7.8.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	902

A.4.7.8.1.1	Test Purpose and Environment	902

A.4.7.8.1.2	Test parameters	903

A.4.7.8.1.3	Test Requirements	906

A.4.7.8.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	906

A.4.7.8.2.1	Test Purpose and Environment	906

A.4.7.8.2.2	Test parameters	907

A.4.7.8.2.3	Test Requirements	910

A.4.7.9	CSI-RSRQ	911

A.4.7.9.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	911

A.4.7.9.1.1	Test Purpose and Environment	911

A.4.7.9.1.2	Test Parameters	911

A.4.7.9.1.3	Test Requirements	916

A.4.7.9.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	916

A.4.7.9.2.1	Test Purpose and Environment	916

A.4.7.9.2.2	Test Parameters	916

A.4.7.2.2.3	Test Requirements	921

A.4.7.10	CSI-SINR	921

A.4.7.10.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	921

A.4.7.10.1.1	Test Purpose and Environment	921

A.4.7.10.1.2	Test Parameters	922

A.4.7.10.1.3	Test Requirements	925

A.4.7.10.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	925

A.4.7.10.2.1	Test Purpose and Environment	925

A.4.7.10.2.2	Test Parameters	925

A.4.7.10.2.3	Test Requirements	928

A.4.8	Void	929

A.5	EN-DC tests with one or more NR cells in FR2	929

A.5.1	Void	929

A.5.2	Void	929

A.5.3	RRC_CONNECTED state mobility	929

A.5.3.1	Void	929

A.5.3.2	RRC Connection Mobility Control	929

A.5.3.2.1	Void	929

A.5.3.2.2	Random Access	929

A.5.3.2.2.1	4-step RA type contention based random access test in FR2 for PSCell/SCell in EN-DC	929

A.5.3.2.2.2	4-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	932

A.5.3.2.2.3	2-step RA type contention based random access test in FR2 for PSCell/SCell in EN-DC	937

A.5.3.2.2.4	2-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	941

A.5.3.2.3	Void	945

A.5.4	Timing	945

A.5.4.1	UE transmit timing	945

A.5.4.1.1	NR UE Transmit Timing Test for FR2	945

A.5.4.1.1.1	Test Purpose and environment	945

A.5.4.1.1.2	Test requirements	948

A.5.4.2	UE timer accuracy	949

A.5.4.3	Timing advance	949

A.5.4.3.1 EN-DC FR2 timing advance adjustment accuracy	949

A.5.4.3.1.1 Test Purpose and Environment	949

A.5.4.3.1.2 Test Parameters	949

A.5.4.3.1.3	Test Requirements	953

A.5.5	Signaling characteristics	953

A.5.5.1	Radio link Monitoring	953

A.5.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	953

A.5.5.1.1.1	Test Purpose and Environment	953

A.5.5.1.1.2	Test Requirements	956

A.5.5.1.2	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	957

A.5.5.1.2.1	Test Purpose and Environment	957

A.5.5.1.2.2	Test Requirements	960

A.5.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	961

A.5.5.1.3.1	Test Purpose and Environment	961

A.5.5.1.3.2	Test Requirements	964

A.5.5.1.4	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	964

A.5.5.1.4.1	Test Purpose and Environment	964

A.5.5.1.4.2	Test Requirements	968

A.5.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	968

A.5.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	974

A.5.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	979

A.5.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	985

A.5.5.1.8.2	Test Requirements	991

A.5.5.1.9	EN-DC Radio Link Monitoring UE Scheduling Restrictions on FR2	991

A.5.5.1.9.1	Test Purpose and Environment	991

A.5.5.1.9.2	Test Requirements	994

A.5.5.2	Interruption	994

A.5.5.2.1	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	994

A.5.5.2.1.1	Test Purpose and Environment	994

A.5.5.2.1.2	Test Requirements	997

A.5.5.2.2	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	997

A.5.5.2.2.1	Test Purpose and Environment	997

A.5.5.2.2.2	Test Requirements	1000

A.5.5.2.3	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	1000

A.5.5.2.3.1	Test Purpose and Environment	1000

A.5.5.2.3.2	Test Requirements	1004

A.5.5.2.4	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	1004

A.5.5.2.4.1	Test Purpose and Environment	1004

A.5.5.2.4.2	Test Requirements	1007

A.5.5.2.5	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	1008

A.5.5.2.5.1	Test Purpose and Environment	1008

A.5.5.2.5.2	Test Requirements	1011

A.5.5.2.6	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	1012

A.5.5.2.6.1	Test Purpose and Environment	1012

A.5.5.2.6.2	Test Requirements	1014

A.5.5.2.7	E-UTRAN – NR FR2 interruptions at E-UTRA SRS carrier based switching	1015

A.5.5.2.7.1	Test Purpose and Environment	1015

A.5.5.2.7.2	Test Requirements	1018

A.5.5.2.8 E-UTRAN – NR FR2 interruptions at NR SRS carrier based switching	1018

A.5.5.2.8.1 Test Purpose and Environment	1018

A.5.5.2.8.3	Test Requirements	1020

A.5.5.3	SCell Activation and Deactivation Delay	1020

A.5.5.3.1	SCell Activation and deactivation of SCell in FR2 intra-band	1020

A.5.5.3.1.1	Test Purpose and Environment	1020

A.5.5.3.1.2	Test Requirements	1023

A.5.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	1023

A.5.5.3.2.1	Test Purpose and Environment	1023

A.5.5.3.2.2	Test Requirements	1027

A.5.5.3.3	Void	1027

A.5.5.3.4	Void	1027

A.5.5.3.5	SCell Activation and deactivation of SCell in FR2	1027

A.5.5.3.5.1	Test Purpose and Environment	1027

A.5.5.3.5.2	Test Requirements	1032

A.5.5.3.6	Multiple SCell Activation and deactivation of one unknown SCell and one known SCell in FR2	1033

A.5.5.3.6.1	Test Purpose and Environment	1033

A.5.5.3.6.2	Test Requirements	1036

A.5.5.3.7	Direct SCell activation at SCell addition of known SCell in FR2	1037

A.5.5.3.7.1	Test Purpose and Environment	1037

A.5.5.3.7.2	Test Requirements	1040

A.5.5.4	Void	1040

A.5.5.5	Beam Failure Detection and Link recovery procedures	1040

A.5.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1040

A.5.5.5.1.1	Test Purpose and Environment	1040

A.5.5.5.1.2	Test Requirements	1046

A.5.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode	1046

A.5.5.5.2.1	Test Purpose and Environment	1046

A.5.5.5.2.2	Test Requirements	1052

A.5.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	1052

A.5.5.5.3.1	Test Purpose and Environment	1052

A.5.5.5.3.2	Test Requirements	1058

A.5.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1058

A.5.5.5.4.1	Test Purpose and Environment	1058

A.5.5.5.4.2	Test Requirements	1064

A.5.5.5.5	EN-DC scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1064

A.5.5.5.5.1	Test Purpose and Environment	1064

A.5.5.5.5.2	Test Requirements	1070

A.5.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	1070

A.5.5.5.6.1	Test Purpose and Environment	1070

A.5.5.5.6.2	Test Requirements	1075

A.5.5.5.7	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	1075

A.5.5.5.7.1	Test Purpose and Environment	1075

A.5.5.5.7.2	Test Requirements	1081

A.5.5.6	Active BWP switch	1081

A.5.5.6.1	DCI-based and Timer-based Active BWP Switch	1081

A.5.5.6.1.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	1081

A.5.5.6.1.1.1	Test Purpose and Environment	1081

A.5.5.6.1.1.2	Test Requirements	1085

A.5.5.6.1.2	E-UTRAN – NR PSCell FR2 with FR2 SCell DL active BWP switch in non-DRX in synchronous EN-DC	1085

A.5.5.6.2	RRC-based Active BWP Switch	1090

A.5.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1094

A.5.5.6.3.1	E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs in synchronous EN-DC	1094

A.5.5.6.3.1.1	Test Purpose and Environment	1094

A.5.5.6.3.1.2	Test Requirements	1098

A.5.5.6.4	SCell dormancy switch	1099

A.5.5.6.4.1	E-UTRAN – NR FR2 PSCell SCell dormancy switch of single FR2 SCell inside active time	1099

A.5.5.6.4.1.1	Test Purpose and Environment	1099

A.5.5.6.4.1.2	Test Requirements	1103

A.5.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR2 SCells outside active time	1103

A.5.5.6.4.2.1	Test Purpose and Environment	1103

A.5.5.6.4.2.2	Test Requirements	1110

A.5.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1110

A.5.5.7	PSCell addition and release delay	1113

A.5.5.7.1	Addition and Release Delay of NR PSCell	1113

A.5.5.7.1.1	Test purpose and environment	1113

A.5.5.7.1.2	Test Requirements	1117

A.5.5.8	Active TCI state switch delay	1118

A.5.5.8.1	MAC-CE based active TCI state switch	1118

A.5.5.8.1.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1118

A.5.5.8.1.1.1	Test Purpose and Environment	1118

A.5.5.8.1.1.2	Test Requirements	1120

A.5.5.8.2	RRC based active TCI state switch	1120

A.5.5.8.2.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1120

A.5.5.8.2.1.1	Test Purpose and Environment	1120

A.5.5.8.2.1.2	Test Requirements	1123

A.5.5.9	Uplink spatial relation switch delay	1123

A.5.5.9.1	MAC-CE based uplink spatial relation switch	1123

A.5.5.9.1.1	E-UTRAN – NR PSCell FR2 uplink spatial relation switch for a known spatial relation	1123

A.5.5.9.1.1.1	Test Purpose and Environment	1123

A.5.5.9.1.1.2	Test Requirements	1126

A.5.5.9.2	RRC based spatial relation switch	1126

A.5.5.9.2.1	E-UTRAN – NR PSCell FR2 spatial relation switch associated with a known DL-RS	1126

A.5.5.9.2.1.1	Test Purpose and Environment	1126

A.5.5.9.2.1.2	Test Requirements	1129

A.5.5.10	UE specific CBW change	1129

A.5.5.10.1	UE specific CBW change on FR2 NR PSCell	1129

A.5.5.10.1.1	Test Purpose and Environment	1129

A.5.5.10.1.2	Test Requirements	1132

A.5.6	Measurement procedure	1133

A.5.6.1	Intra-frequency Measurements	1133

A.5.6.1.1	EN-DC event triggered reporting test without gap under non-DRX	1133

A.5.6.1.1.1	Test purpose and Environment	1133

A.5.6.1.1.2	Test Requirements	1137

A.5.6.1.2	EN-DC event triggered reporting test without gap under DRX	1137

A.5.6.1.2.1	Test purpose and Environment	1137

A.5.6.1.2.2	Test Requirements	1139

A.5.6.1.3	EN-DC event triggered reporting test with per-UE gaps under non-DRX	1140

A.5.6.1.3.1	Test purpose and Environment	1140

A.5.6.1.3.2	Test Requirements	1144

A.5.6.1.4	EN-DC event triggered reporting test with per-UE gaps under DRX	1144

A.5.6.1.4.1	Test purpose and Environment	1144

A.5.6.1.4.2	Test Requirements	1147

A.5.6.2	Inter-frequency Measurements	1148

A.5.6.2.1 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	1148

A.5.6.2.1.1	Test Purpose and Environment	1148

A.5.6.2.1.2	Test Requirements	1151

A.5.6.2.2 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	1151

A.5.6.2.2.1	Test Purpose and Environment	1151

A.5.6.2.2.2	Test Requirements	1155

A.5.6.2.3 	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	1155

A.5.6.2.3.1	Test Purpose and Environment	1155

A.5.6.2.3.2	Test Requirements	1159

A.5.6.2.4	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	1159

A.5.6.2.4.1	Test Purpose and Environment	1159

A.5.6.2.4.2	Test Requirements	1163

A.5.6.2.5	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	1163

A.5.6.2.5.1	Test Purpose and Environment	1163

A.5.6.2.5.2	Test Requirements	1168

A.5.6.2.6	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	1168

A.5.6.2.6.1	Test Purpose and Environment	1168

A.5.6.2.6.2	Test Requirements	1172

A.5.6.2.7	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	1172

A.5.6.2.7.1	Test Purpose and Environment	1172

A.5.6.2.7.2	Test Requirements	1177

A.5.6.2.8	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	1177

A.5.6.2.8.1	Test Purpose and Environment	1177

A.5.6.2.8.2	Test Requirements	1182

A.5.6.3	L1-RSRP measurement for beam reporting	1182

A.5.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1182

A.5.6.3.1.1	Test Purpose and Environment	1182

A.5.6.3.1.2	Test parameters	1182

A.5.6.3.1.3	Test Requirements	1184

A.5.6.3.2	SSB based L1-RSRP measurement when DRX is used	1184

A.5.6.3.2.1	Test Purpose and Environment	1184

A.5.6.3.2.2	Test parameters	1185

A.5.6.3.2.3	Test Requirements	1187

A.5.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	1187

A.5.6.3.3.1	Test Purpose and Environment	1187

A.5.6.3.3.2	Test parameters	1188

A.5.6.3.3.3	Test Requirements	1190

A.5.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	1191

A.5.6.3.4.1	Test Purpose and Environment	1191

A.5.6.3.4.2	Test parameters	1191

A.5.6.3.4.3	Test Requirements	1193

A.5.6.4	CLI measurements	1194

A.5.6.4.1	SRS-RSRP measurement with DRX	1194

A.5.6.4.1.1	Test Purpose and Environment	1194

A.5.6.4.1.2	Test Parameters	1194

A.5.6.4.1.3	Test Requirements	1196

A.5.6.4.2	CLI-RSSI measurement with DRX	1196

A.5.6.4.2.1	Test Purpose and Environment	1196

A.5.6.4.2.2	Test Parameters	1197

A.5.6.4.2.3	Test Requirements	1199

A.5.6.5	Measurements with autonomous gaps	1199

A.5.6.5.1 	EN-DC inter-frequency CGI identification of NR neighbor cell in FR2	1199

A.5.6.5.1.1	Test Purpose and Environment	1199

A.5.6.5.1.2	Test Requirements	1202

A.5.6.6	L1-SINR measurement for beam reporting	1203

A.5.6.6.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is used	1203

A.5.6.6.1.1	Test Purpose and Environment	1203

A.5.6.6.1.2	Test parameters	1203

A.5.6.6.1.3	Test Requirements	1205

A.5.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	1205

A.5.6.6.2.1	Test Purpose and Environment	1205

A.5.6.6.2.2	Test parameters	1206

A.5.6.6.2.3	Test Requirements	1209

A.5.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	1209

A.5.6.6.3.1	Test Purpose and Environment	1209

A.5.6.6.3.2	Test parameters	1209

A.5.6.6.3.3	Test Requirements	1211

A.5.6.7	CSI-RS based Intra-frequency Measurements	1211

A.5.6.7.1	EN-DC event triggered reporting test without gap under non-DRX	1211

A.5.6.7.1.1	Test purpose and Environment	1211

A.5.6.7.1.2	Test Requirements	1214

A.5.6.8	CSI-RS based Inter-frequency Measurements	1214

A.5.6.8.1 	EN-DC event triggered reporting tests for NR FR2 cell when DRX is used	1214

A.5.6.8.1.1	Test Purpose and Environment	1214

A.5.6.8.1.2	Test Requirements	1218

A.5.7	Measurement Performance requirements	1218

A.5.7.1	SS-RSRP	1219

A.5.7.1.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1219

A.5.7.1.1.1	Test Purpose and Environment	1219

A.5.7.1.1.2	Test parameters	1219

A.5.7.1.1.3	Test Requirements	1221

A.5.7.1.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1222

A.5.7.1.2.1	Test Purpose and Environment	1222

A.5.7.1.2.2	Test parameters	1222

A.5.7.1.2.3	Test Requirements	1226

A.5.7.1.3	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	1227

A.5.7.1.3.1	Test Purpose and Environment	1227

A.5.7.1.3.2	Test parameters	1227

A.5.7.1.3.3	Test Requirements	1230

A.5.7.2	SS-RSRQ	1231

A.5.7.2.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1231

A.5.7.2.1.1	Test Purpose and Environment	1231

A.5.7.2.1.2	Test Parameters	1231

A.5.7.2.1.3	Test Requirements	1233

A.5.7.2.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1234

A.5.7.2.2.1	Test Purpose and Environment	1234

A.5.7.2.2.2	Test Parameters	1234

A.5.7.2.2.3	Test Requirements	1236

A.5.7.3	SS-SINR	1236

A.5.7.3.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1236

A.5.7.3.1.1	Test Purpose and Environment	1236

A.5.7.3.1.2	Test Parameters	1236

A.5.7.3.1.3	Test Requirements	1238

A.5.7.3.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1238

A.5.7.3.2.1	Test Purpose and Environment	1238

A.5.7.3.2.2	Test Parameters	1238

A.5.7.3.2.3	Test Requirements	1240

A.5.7.4	L1-RSRP measurement for beam reporting	1240

A.5.7.4.1	SSB based L1-RSRP measurement	1240

A.5.7.4.1.1	Test Purpose and Environment	1240

A.5.7.4.1.2	Test parameters	1241

A.5.7.4.1.3	Test Requirements	1243

A.5.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1244

A.5.7.4.2.1	Test Purpose and Environment	1244

A.5.7.4.2.2	Test parameters	1244

A.5.7.4.2.3	Test Requirements	1246

A.5.7.5	CLI measurements	1247

A.5.7.5.1	EN-DC SRS-RSRP measurement accuracy with FR2 serving cell	1247

A.5.7.5.1.1	Test Purpose and Environment	1247

A.5.7.5.1.2	Test parameters	1247

A.5.7.5.1.3	Test Requirements	1250

A.5.7.5.2	EN-DC CLI-RSSI measurement accuracy with FR2 serving cell	1251

A.5.7.5.2.1	Test Purpose and Environment	1251

A.5.7.5.2.2	Test parameters	1251

A.5.7.5.2.3	Test Requirements	1253

A.5.7.6	L1-SINR measurement for beam reporting	1254

A.5.7.6.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	1254

A.5.7.6.1.1	Test Purpose and Environment	1254

A.5.7.6.1.2	Test parameters	1254

A.5.7.6.1.3	Test Requirements	1256

A.5.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	1256

A.5.7.6.2.1	Test Purpose and Environment	1256

A.5.7.6.2.2	Test parameters	1257

A.5.7.6.2.3	Test Requirements	1259

A.5.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	1260

A.5.7.6.3.1	Test Purpose and Environment	1260

A.5.7.6.3.2	Test parameters	1260

A.5.7.6.3.3	Test Requirements	1262

A.5.7.7	CSI-RSRP	1262

A.5.7.7.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1262

A.5.7.7.1.2	Test parameters	1263

A.5.7.7.1.3	Test Requirements	1265

A.5.7.7.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1266

A.5.7.7.2.1	Test Purpose and Environment	1266

A.5.7.7.2.2	Test parameters	1266

A.5.7.7.2.3	Test Requirements	1270

A.5.7.8	CSI-RSRQ	1271

A.5.7.8.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	1271

A.5.7.8.1.1	Test Purpose and Environment	1271

A.5.7.8.1.2	Test Parameters	1271

A.5.7.8.1.3	Test Requirements	1273

A.5.7.8.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1273

A.5.7.8.2.1	Test Purpose and Environment	1273

A.5.7.8.2.2	Test Parameters	1273

A.5.7.9	CSI-SINR	1275

A.5.7.9.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1275

A.5.7.9.1.1	Test Purpose and Environment	1275

A.5.7.9.1.2	Test Parameters	1275

Table A.5.7.9.1.2-1: CSI-SINR Intra frequency CSI-SINR supported test configurations	1276

A.5.7.9.1.3	Test Requirements	1278

A.5.7.9.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1278

A.5.7.9.2.1	Test Purpose and Environment	1278

A.5.7.9.2.2	Test Parameters	1278

A.5.7.9.2.3	Test Requirements	1281

A.5.8	Void	1281

A.6	NR standalone tests with all NR cells in FR1	1011

A.6.1	SA: RRC_IDLE state mobility	1011

A.6.1.1	Cell re-selection to NR	1011

A.6.1.1.1	Cell reselection to FR1 intra-frequency NR case	1011

A.6.1.1.1.1	Test Purpose and Environment	1011

A.6.1.1.1.2	Test Parameters	1011

A.6.1.1.1.3	Test Requirements	1015

A.6.1.1.2	Cell reselection to FR1 inter-frequency NR case	1015

A.6.1.1.2.1	Test Purpose and Environment	1015

A.6.1.1.2.2	Test Parameters	1015

A.6.1.1.2.3	Test Requirements	1019

A.6.1.1.3	Cell reselection to FR1 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1019

A.6.1.1.3.1	Test Purpose and Environment	1019

A.6.1.1.3.2	Test Parameters	1019

A.6.1.1.3.3	Test Requirements	1024

A.6.1.1.4	Cell reselection to FR1 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1024

A.6.1.1.4.1	Test Purpose and Environment	1024

A.6.1.1.4.2	Test Parameters	1024

A.6.1.1.4.3	Test Requirements	1028

A.6.1.1.5	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1028

A.6.1.1.5.1	Test Purpose and Environment	1028

A.6.1.1.5.2	Test Parameters	1028

A.6.1.1.5.3	Test Requirements	1032

A.6.1.1.6	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1033

A.6.1.1.6.1	Test Purpose and Environment	1033

A.6.1.1.6.2	Test Parameters	1033

A.6.1.1.6.3	Test Requirements	1037

A.6.1.1.7	Cell reselection to FR1 intra-frequency NR case for UE configured with highSpeedMeasFlag-r16	1038

A.6.1.1.7.1	Test Purpose and Environment	1038

A.6.1.1.7.2	Test Parameters	1038

A.6.1.1.7.3	Test Requirements	1042

A.6.1.2	Inter-RAT E-UTRAN cell re-selection	1042

A.6.1.2.1	Cell reselection to higher priority E-UTRAN	1042

A.6.1.2.1.1	Test Purpose and Environment	1042

A.6.1.2.1.2	Test Parameters	1042

A.6.1.2.1.3	Test Requirements	1045

A.6.1.2.2	Cell reselection to lower priority E-UTRAN	1046

A.6.1.2.2.1	Test Purpose and Environment	1046

A.6.1.2.2.2	Test Parameters	1046

A.6.1.2.2.3	Test Requirements	1049

A.6.1.2.3	Cell reselection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion	1050

A.6.1.2.3.1	Test Purpose and Environment	1050

A.6.1.2.3.2	Test Parameters	1050

A.6.1.2.3.3	Test Requirements	1053

A.6.1.2.4	Cell reselection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion	1054

A.6.1.2.4.1	Test Purpose and Environment	1054

A.6.1.2.4.2	Test Parameters	1054

A.6.1.2.4.3	Test Requirements	1057

A.6.1.2.5.3	Test Requirements	1061

A.6.2	SA: RRC_INACTIVE state mobility	1062

A.6.3	RRC_CONNECTED state mobility	1062

A.6.3.1	Handover	1062

A.6.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell	1062

A.6.3.1.1.1	Test Purpose and Environment	1062

A.6.3.1.1.2	Test Parameters	1062

A.6.3.1.1.3 Test Requirements	1066

A.6.3.1.2	Intra-frequency handover from FR1 to FR1; unknown target cell	1066

A.6.3.1.2.1	Test Purpose and Environment	1066

A.6.3.1.2.2	Test Parameters	1066

A.6.3.1.2.3	Test Requirements	1070

A.6.3.1.3	Inter-frequency handover from FR1 to FR1; unknown target cell	1070

A.6.3.1.3.1	Test Purpose and Environment	1070

A.6.3.1.3.2	Test Parameters	1070

A.6.3.1.3.3 Test Requirements	1074

A.6.3.1.4	 SA NR - E-UTRAN handover	1074

A.6.3.1.4.1	Test Purpose and Environment	1074

A.6.3.1.4.2	Test Requirements	1080

A.6.3.1.5	SA NR - E-UTRAN handover with unknown target cell	1080

A.6.3.1.5.1	Test Purpose and Environment	1080

A.6.3.1.5.2	Test Requirements	1086

A.6.3.1.6	 SA NR - UTRAN FDD handover	1086

A.6.3.1.6.1	Test Purpose and Environment	1086

A.6.3.1.6.2	Test Requirements	1090

A.6.3.1.7	Intra-frequency handover in FR1; synchronous scenario	1090

A.6.3.1.7.1	Test Purpose and Environment	1090

A.6.3.1.7.2	Test Parameters	1090

A.6.3.1.7.3 Test Requirements	1094

A.6.3.1.8	Intra-frequency handover in FR1; asynchronous scenario	1095

A.6.3.1.8.1	Test Purpose and Environment	1095

A.6.3.1.8.2	Test Parameters	1095

A.6.3.1.8.3 Test Requirements	1099

A.6.3.1.9	Intra-band inter-frequency synchronous DAPS handover test in SA for FR1	1100

A.6.3.1.9.1	Test Purpose and Environment	1100

A.6.3.1.9.2	Test Parameters	1100

A.6.3.1.9.3 Test Requirements	1104

A.6.3.1.10	Intra-band inter-frequency asynchronous DAPS handover test in SA for FR1	1104

A.6.3.1.10.1	Test Purpose and Environment	1104

A.6.3.1.10.2	Test Parameters	1104

A.6.3.1.10.3 Test Requirements	1108

A.6.3.1.11	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR1	1108

A.6.3.1.11.1	Test Purpose and Environment	1108

A.6.3.1.11.2	Test Parameters	1108

A.6.3.1.11.3 Test Requirements	1115

A.6.3.1.12	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR1	1115

A.6.3.1.12.1	Test Purpose and Environment	1115

A.6.3.1.12.2	Test Parameters	1115

A.6.3.1.12.3 Test Requirements	1122

A.6.3.2	RRC Connection Mobility Control	1122

A.6.3.2.1	SA: RRC Re-establishment	1122

A.6.3.2.1.1	Intra-frequency RRC Re-establishment in FR1	1122

A.6.3.2.1.2	Inter-frequency RRC Re-establishment in FR1	1126

A.6.3.2.1.3	Intra-frequency RRC Re-establishment in FR1 without serving cell timing	1130

A.6.3.2.2	Random Access	1133

A.6.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	1133

A.6.3.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone	1138

A.6.3.2.2.3	2-step RA type contention based random access test in FR1 for NR standalone	1142

A.6.3.2.2.4	2-step RA type non-contention based test in FR1 for NR standalone	1147

A.6.3.2.3	SA: RRC Connection Release with Redirection	1151

A.6.3.2.3.1	Redirection from NR in FR1 to NR in FR1	1151

A.6.3.2.3.2	Redirection from NR in FR1 to E-UTRAN	1155

A.6.3.3 Conditional handover	1162

A.6.3.3.1	Intra-frequency conditional handover from FR1 to FR1	1162

A.6.3.3.1.1	Test Purpose and Environment	1162

A.6.3.3.1.2	Test Parameters	1162

A.6.3.3.1.3 Test Requirements	1166

A.6.3.3.2	Inter-frequency conditional handover from FR1 to FR1	1166

A.6.3.3.2.1	Test Purpose and Environment	1166

A.6.3.3.2.2	Test Parameters	1166

A.6.3.3.2.3 Test Requirements	1170

A.6.4	Timing	1170

A.6.4.1	UE transmit timing	1170

A.6.4.1.1	NR UE Transmit Timing Test for FR1	1170

A.6.4.1.1.1	Test Purpose and environment	1170

A.6.4.1.1.2	Test requirements	1174

A.6.4.2	UE timer accuracy	1174

A.6.4.3	Timing advance	1174

A.6.4.3.1	SA FR1 timing advance adjustment accuracy	1174

A.6.4.3.1.1	Test Purpose and Environment	1174

A.6.4.3.1.2	Test Parameters	1174

A.6.4.3.1.3	Test Requirements	1178

A.6.5	Signalling characteristics	1178

A.6.5.1	Radio link Monitoring	1178

A.6.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	1179

A.6.5.1.1.1	Test Purpose and Environment	1179

A.6.5.1.1.2	Test Requirements	1183

A.6.5.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	1183

A.6.5.1.2.1	Test Purpose and Environment	1183

A.6.5.1.2.2	Test Requirements	1189

A.6.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	1189

A.6.5.1.3.1	Test Purpose and Environment	1189

A.6.5.1.3.2	Test Requirements	1195

A.6.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	1195

A.6.5.1.4.1	Test Purpose and Environment	1195

A.6.5.1.4.2	Test Requirements	1201

A.6.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	1201

A.6.5.1.5.1	Test Purpose and Environment	1201

A.6.5.1.5.2	Test Requirements	1206

A.6.5.1.6	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	1206

A.6.5.1.6.1	Test Purpose and Environment	1206

A.6.5.1.6.2	Test Requirements	1212

A.6.5.1.7	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	1212

A.6.5.1.7.1	Test Purpose and Environment	1212

A.6.5.1.7.2	Test Requirements	1216

A.6.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	1216

A.6.5.1.8.1	Test Purpose and Environment	1216

A.6.5.1.8.2	Test Requirements	1222

A.6.5.2	Interruption	1222

A.6.5.2.1	Interruptions during measurements on deactivated NR SCC in FR1	1222

A.6.5.2.1.2	Test Requirements	1226

A.6.5.2.2	SA interruptions at NR SRS carrier based switching	1227

A.6.5.2.2.1	Test Purpose and Environment	1227

A.6.5.2.2.2	Test Parameters	1227

A.6.5.2.2.3	Test Requirements	1229

A.6.5.3	SCell Activation and Deactivation Delay	1230

A.6.5.3.1	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle	1230

A.6.5.3.1.1	Test Purpose and Environment	1230

A.6.5.3.1.2	Test Requirements	1234

A.6.5.3.2	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 320ms SCell measurement cycle	1235

A.6.5.3.2.1	Test Purpose and Environment	1235

A.6.5.3.2.2	Test Requirements	1235

A.6.5.3.3	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	1235

A.6.5.3.3.1	Test Purpose and Environment	1235

A.6.5.3.3.2	Test Requirements	1236

A.6.5.3.4	Direct SCell activation at SCell addition of known SCell in FR1	1236

A.6.5.3.4.1	 Test Purpose and Environment	1236

A.6.5.3.4.2	 Test Requirements	1240

A.6.5.3.5	Direct SCell activation at handover with known SCell in FR1	1240

A.6.5.3.5.1	Test Purpose and Environment	1240

A.6.5.3.5.2	Test Requirements	1244

A.6.5.4	UE UL carrier RRC reconfiguration Delay	1245

A.6.5.4.1	UE UL carrier RRC reconfiguration Delay	1245

A.6.5.4.1.1	Test Purpose and Environment	1245

A.6.5.4.1.2	Test Requirements	1255

A.6.5.4.2	Void	1256

A.6.5.5	Beam Failure Detection and Link recovery procedures	1256

A.6.5.5.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	1256

A.6.5.5.1.1	Test Purpose and Environment	1256

A.6.5.5.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	1261

A.6.5.5.2.1	Test Purpose and Environment	1261

A.6.5.5.2.2	Test Requirements	1268

A.6.5.5.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	1268

A.6.5.5.3.1	Test Purpose and Environment	1268

A.6.5.5.3.2	Test Requirements	1274

A.6.5.5.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	1274

A.6.5.5.4.1	Test Purpose and Environment	1274

A.6.5.5.4.2	Test Requirements	1280

A.6.5.5.5	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	1280

A.6.5.5.5.1	Test Purpose and Environment	1280

A.6.5.5.5.2	Test Requirements	1284

A.6.5.5.6	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	1285

A.6.5.5.6.1	Test Purpose and Environment	1285

A.6.5.5.6.2	Test Requirements	1289

A.6.5.6	Active BWP switch	1289

A.6.5.6.1	DCI-based and Timer-based Active BWP Switch	1289

A.6.5.6.1.1	NR FR1- NR FR1 DL active BWP switch of SCell with non-DRX in SA	1289

A.6.5.6.1.2	NR FR1 DL active BWP switch with non-DRX in SA	1295

A.6.5.6.2	RRC-based Active BWP Switch	1299

A.6.5.6.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	1299

A.6.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1303

A.6.5.6.3.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	1303

A.6.5.6.3.1.1	Test Purpose and Environment	1303

A.6.5.6.3.1.2	Test Requirements	1308

A.6.5.6.4	SCell dormancy switch	1309

A.6.5.6.4.1	NR FR1 PCell SCell dormancy switch of single FR1 SCell outside active time	1309

A.6.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR1 SCells inside active time	1315

A.6.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1319

A.6.5.6.5.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	1319

A.6.5.7	DL interruptions at switching between two uplink carriers	1323

A.6.5.7.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	1323

A.6.5.7.1.1	Test Purpose and Environment	1323

A.6.5.7.1.2	Test Requirements	1327

A.6.5.7.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	1327

A.6.5.7.2.1	Test Purpose and Environment	1327

A.6.5.7.2.2	Test Requirements	1331

A.6.5.8	UE specific CBW change	1331

A.6.5.8.1	UE specific CBW change on PCell in FR1 in non-DRX	1331

A.6.5.8.1.1	Test Purpose and Environment	1331

A.6.5.8.1.2	Test Requirements	1335

A.6.6	Measurement procedure	1335

A.6.6.1	Intra-frequency Measurements	1335

A.6.6.1.1	SA event triggered reporting tests without gap under non-DRX	1335

A.6.6.1.1.1	Test purpose and Environment	1335

A.6.6.1.1.2	Test parameters	1335

A.6.6.1.1.3	Test Requirements	1339

A.6.6.1.2	SA event triggered reporting tests without gap under DRX	1339

A.6.6.1.2.1	Test purpose and Environment	1339

A.6.6.1.2.2	Test parameters	1339

A.6.6.1.2.3	Test Requirements	1343

A.6.6.1.3	SA event triggered reporting tests with per-UE gaps under non-DRX	1343

A.6.6.1.3.1	Test purpose and Environment	1343

A.6.6.1.3.2	Test parameters	1343

A.6.6.1.3.3	Test Requirements	1347

A.6.6.1.4	SA event triggered reporting tests with per-UE gaps under DRX	1347

A.6.6.1.4.1	Test purpose and Environment	1347

A.6.6.1.4.2	Test parameters	1347

A.6.6.1.4.3	Test Requirements	1351

A.6.6.1.5	SA event triggered reporting tests without gap under non-DRX with SSB index reading	1351

A.6.6.1.5.1	Test purpose and Environment	1351

A.6.6.1.5.2	Test parameters	1351

A.6.6.1.5.3	Test Requirements	1353

A.6.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading	1354

A.6.6.1.6.1	Test purpose and Environment	1354

A.6.6.1.6.2	Test parameters	1354

A.6.6.1.6.3	Test Requirements	1355

A.6.6.1.7	SA event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	1356

A.6.6.1.7.1	Test purpose and Environment	1356

A.6.6.1.7.2	Test parameters	1356

A.6.6.1.7.3	Test Requirements	1360

A.6.6.2	Inter-frequency Measurements	1360

A.6.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	1360

A.6.6.2.1.1	Test Purpose and Environment	1360

A.6.6.2.1.2	Test Requirements	1364

A.6.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used	1364

A.6.6.2.2.1	Test Purpose and Environment	1364

A.6.6.2.2.2	Test Requirements	1369

A.6.6.2.3	Void	1370

A.6.6.2.4	Void	1370

A.6.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	1370

A.6.6.2.5.1	Test Purpose and Environment	1370

A.6.6.2.5.2	Test Requirements	1374

A.6.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used	1374

A.6.6.2.6.1	Test Purpose and Environment	1374

A.6.6.2.6.2	Test Requirements	1379

A.6.6.2.7	Void	1380

A.6.6.2.8	Void	1380

A.6.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern	1380

A.6.6.2.9.1	Test Purpose and Environment	1380

A.6.6.2.9.2	Test Requirements	1383

A.6.6.2.10	SA event triggered reporting tests for FR1 when DRX is used	1383

A.6.6.2.10.1	Test Purpose and Environment	1383

A.6.6.2.10.2	Test Requirements	1387

A.6.6.2.11	SA event triggered reporting tests for FR1 without gap when DRX is not used	1388

A.6.6.2.11.1	Test Purpose and Environment	1388

A.6.6.2.11.2	Test Requirements	1391

A.6.6.3	Inter-RAT Measurements	1391

A.6.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	1391

A.6.6.3.1.1	Test Purpose and Environment	1391

A.6.6.3.1.2	Test Requirements	1397

A.6.6.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	1397

A.6.6.3.2.1	Test Purpose and Environment	1397

A.6.6.3.2.2	Test Requirements	1404

A.6.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for UE configured with highSpeedMeasFlag-r16	1404

A.6.6.3.3.1	Test Purpose and Environment	1404

A.6.6.3.3.2	Test Requirements	1411

A.6.6.4	L1-RSRP measurement for beam reporting	1411

A.6.6.4.1	SSB based L1-RSRP measurement when DRX is not used	1411

A.6.6.4.1.1	Test Purpose and Environment	1411

A.6.6.4.1.2	Test parameters	1411

A.6.6.4.1.3	Test Requirements	1414

A.6.6.4.2	SSB based L1-RSRP measurement when DRX is used	1414

A.6.6.4.2.1	Test Purpose and Environment	1414

A.6.6.4.2.2	Test parameters	1415

A.6.6.4.2.3	Test Requirements	1418

A.6.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	1418

A.6.6.4.3.1	Test Purpose and Environment	1418

A.6.6.4.3.2	Test parameters	1419

A.6.6.4.3.3	Test Requirements	1422

A.6.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	1422

A.6.6.4.4.1	Test Purpose and Environment	1422

A.6.6.4.4.2	Test parameters	1423

A.6.6.4.4.3	Test Requirements	1426

A.6.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	1426

A.6.6.4.5.1	Test Purpose and Environment	1426

A.6.6.4.5.2	Test parameters	1427

A.6.6.4.5.3	Test Requirements	1430

A.6.6.5	1430

A.6.6.5.1	SA NR - UTRAN FDD event-triggered reporting in non-DRX in FR1	1430

A.6.6.5.1.1	Test Purpose and Environment	1430

A.6.6.5.1.2	Test Requirements	1434

A.6.6.6	CLI measurements	1434

A.6.6.6.1	SRS-RSRP measurement with DRX	1434

A.6.6.6.1.1	Test Purpose and Environment	1434

A.6.6.6.1.2	Test Parameters	1435

A.6.6.6.1.3	Test Requirements	1438

A.6.6.6.2	CLI-RSSI measurement with DRX	1438

A.6.6.6.2.1	Test Purpose and Environment	1438

A.6.6.6.2.2	Test Parameters	1439

A.6.6.6.2.3	Test Requirements	1441

A.6.6.7	NR measurements with autonomous gaps	1441

A.6.6.7.1	SA intra-frequency CGI identification of NR neighbor cell in FR1	1441

A.6.6.7.1.1	Test Purpose and Environment	1441

A.6.6.7.1.2	Test Parameters	1441

A.6.6.7.1.3	Test Requirements	1444

A.6.6.7.2	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA	1444

A.6.6.7.2.1	Test Purpose and Environment	1444

A.6.6.7.2.2	Test Requirements	1447

A.6.6.8	L1-SINR measurement for beam reporting	1448

A.6.6.8.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is used	1448

A.6.6.8.1.1	Test Purpose and Environment	1448

A.6.6.8.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	1451

A.6.6.8.2.1	Test Purpose and Environment	1451

A.6.6.8.2.2	Test parameters	1452

A.6.6.8.2.3	Test Requirements	1456

A.6.6.8.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	1456

A.6.6.8.3.1	Test Purpose and Environment	1456

A.6.6.8.3.2	Test parameters	1457

A.6.6.8.3.3	Test Requirements	1460

A.6.6.9	Idle Mode CA/DC Measurements	1460

A.6.6.9.1	SA Idle mode CA/DC measurement for FR1	1460

A.6.6.9.1.1	Test Purpose and Environment	1460

A.6.6.9.1.2	Test Requirements	1467

A.6.6.10	CSI-RS based intra-frequency Measurements	1467

A.6.6.10.1	SA event triggered reporting tests without gap under non-DRX	1467

A.6.6.10.1.1	Test purpose and Environment	1467

A.6.6.10.1.2	Test Requirements	1470

A.6.6.11	CSI-RS based inter-frequency Measurements	1470

A.6.6.11.1	 SA event triggered reporting tests with gap under DRX	1470

A.6.6.11.1.1	Test Purpose and Environment	1470

A.6.6.11.1.2	Test Requirements	1474

A.6.6.12	RSTD measurements	1474

A. 6.6.12.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA	1474

A. 6.6.12.1.1	Test Purpose and Environment	1474

A.6.6.12.1.2	Test Requirements	1480

A. 6.6.12.2	NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR1 SA	1480

A. 6.6.12.2.1	Test Purpose and Environment	1480

A.6.6.12.2.2	Test Requirements	1486

A.6.6.13 PRS-RSRP measurements	1486

A.6.6.13.1	PRS-RSRP reporting delay test case for single positioning frequency layer	1486

A.6.6.13.1.1	Test purpose and Environment	1486

A.6.6.13.1.2	Test Requirements	1490

A.6.6.13.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	1490

A.6.6.13.2.1	Test purpose and Environment	1490

A.6.6.13.2.2	Test Requirements	1494

A.6.6.14	UE Rx-Tx time difference measurements	1494

A.6.6.14.1 UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	1494

A.6.6.14.1.1	Test purpose and environment	1494

A.6.6.14.1.2	Test requirements	1497

A.6.6.14.2 UE Rx-Tx time difference measurement for dual positioning frequency layers in FR1 SA	1497

A.6.6.14.2.1	Test purpose and environment	1497

A.6.6.14.2.2	Test requirements	1500

A.6.6.15	Idle Mode measurements of inter-RAT CA candidate cells for early reporting	1500

A.6.6.15.1	Test Purpose and Environment	1500

A.6.6.15.2	Test Requirements	1508

A.6.7	Measurement Performance requirements	1508

A.6.7.1	SS-RSRP	1509

A.6.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1509

A.6.7.1.1.1	Test Purpose and Environment	1509

A.6.7.1.1.2	Test parameters	1509

A.6.7.1.1.3	Test Requirements	1514

A.6.7.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1514

A.6.7.1.2.1	Test Purpose and Environment	1514

A.6.7.1.2.2	Test parameters	1514

A.6.7.1.2.3	Test Requirements	1518

A.6.7.1.3	Void	1518

A.6.7.2	SS-RSRQ	1518

A.6.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1518

A.6.7.2.1.1	Test Purpose and Environment	1518

A.6.7.2.1.2	Test Parameters	1519

A.6.7.2.1.3	Test Requirements	1524

A.6.7.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1524

A.6.7.2.2.1	Test Purpose and Environment	1524

A.6.7.2.2.2	Test Parameters	1524

A.6.7.2.2.3	Test Requirements	1529

A.6.7.3	SS-SINR	1529

A.6.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1529

A.6.7.3.1.1	Test Purpose and Environment	1529

A.6.7.3.1.2	Test Parameters	1530

A.6.7.3.1.3	Test Requirements	1534

A.6.7.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1534

A.6.7.3.2.1	Test Purpose and Environment	1534

A.6.7.3.2.2	Test Parameters	1534

A.6.7.3.2.3	Test Requirements	1540

A.6.7.4	L1-RSRP measurement for beam reporting	1541

A.6.7.4.1	SSB based L1-RSRP measurement	1541

A.6.7.4.1.1	Test Purpose and Environment	1541

A.6.7.4.1.2	Test parameters	1541

A.6.7.4.1.3	Test Requirements	1545

A.6.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1545

A.6.7.4.2.1	Test Purpose and Environment	1545

A.6.7.4.2.2	Test parameters	1546

A.6.7.4.2.3	Test Requirements	1550

A.6.7.5	E-UTRAN RSRP	1550

A.6.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell	1550

A.6.7.5.1.1	Test Purpose and Environment	1550

A.6.7.5.1.2	Test parameters	1551

A.6.7.5.1.3	Test Requirements	1556

A.6.7.6	E-UTRAN RSRQ	1557

A.6.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell	1557

A.6.7.6.1.1	Test Purpose and Environment	1557

A.6.7.6.1.2	Test parameters	1557

A.6.7.6.1.3	Test Requirements	1562

A.6.7.7	E-UTRAN RS-SINR	1563

A.6.7.7.1	SA: inter-RAT measurement accuracy with FR1 serving cell	1563

A.6.7.7.1.1	Test Purpose and Environment	1563

A.6.7.7.1.2	Test parameters	1563

A.6.7.7.1.3	Test Requirements	1568

A.6.7.8	CLI measurements	1569

A.6.7.8.1	SA SRS-RSRP measurement accuracy with FR1 serving cell	1569

A.6.7.8.1.1	Test Purpose and Environment	1569

A.6.7.8.1.2	Test parameters	1569

A.6.7.8.1.3	Test Requirements	1575

A.6.7.8.2	SA CLI-RSSI measurement accuracy with FR1 serving cell	1575

A.6.7.8.2.1	Test Purpose and Environment	1575

A.6.7.8.2.2	Test parameters	1576

A.6.7.8.2.3	Test Requirements	1579

A.6.7.9	L1-SINR measurement for beam reporting	1580

A.6.7.9.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	1580

A.6.7.9.1.1	Test Purpose and Environment	1580

A.6.7.9.1.2	Test parameters	1580

A.6.7.9.1.3	Test Requirements	1584

A.6.7.9.2	L1-SINR measurement with SSB based CMR and dedicated IMR	1584

A.6.7.9.2.1	Test Purpose and Environment	1584

A.6.7.9.2.2	Test parameters	1585

A.6.7.9.2.3	Test Requirements	1590

A.6.7.9.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	1590

A.6.7.9.3.1	Test Purpose and Environment	1590

A.6.7.9.3.2	Test parameters	1590

A.6.7.9.3.3	Test Requirements	1595

A.6.7.10	CSI-RSRP	1595

A.6.7.10.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1595

A.6.7.10.1.1	Test Purpose and Environment	1595

A.6.7.10.1.2	Test parameters	1596

A.6.7.10.1.3	Test Requirements	1601

A.6.7.10.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1601

A.6.7.10.2.1	Test Purpose and Environment	1601

A.6.7.10.2.2	Test parameters	1601

A.6.7.10.2.3	Test Requirements	1606

A.6.7.11	CSI-RSRQ	1606

A.6.7.11.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1606

A.6.7.11.1.1	Test Purpose and Environment	1606

A.6.7.11.1.2	Test Parameters	1606

A.6.7.11.1.3	Test Requirements	1611

A.6.7.11.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1611

A.6.7.11.2.1	Test Purpose and Environment	1611

A.6.7.11.2.2	Test Parameters	1611

A.6.7.11.2.3	Test Requirements	1616

A.6.7.12	CSI-SINR	1617

A.6.7.12.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1617

A.6.7.12.1.1	Test Purpose and Environment	1617

A.6.7.12.1.2	Test Parameters	1617

A.6.7.12.1.3	Test Requirements	1621

A.6.7.12.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1622

A.6.7.12.2.1	Test Purpose and Environment	1622

A.6.7.12.2.2	Test Parameters	1622

A.6.7.12.2.3	Test Requirements	1628

A.6.7.13	RSTD measurements	1628

A.6.7.13.1	RSTD measurement accuracy test case for single positioning frequency layer	1628

A.6.7.13.1.1	Test purpose and Environment	1628

A.6.7.13.1.2	Test Requirements	1631

A.6.7.13.2	RSTD measurement accuracy test case for dual positioning frequency layer	1631

A.6.7.13.2.1	Test purpose and Environment	1631

A.6.7.13.2.2	Test Requirements	1635

A.6.7.14	PRS-RSRP measurements	1635

A.6.7.14.1	SA: measurement accuracy with PRS in FR1	1635

A.6.7.14.1.1	Test Purpose and Environment	1635

A.6.7.14.1.2	Test parameters	1635

A.6.7.14.1.3	Test Requirements	1641

A.6.7.15	UE Rx-Tx time difference measurements	1641

A.6.7.15.1 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA	1641

A.6.7.15.1.1	Test purpose and environment	1641

A.6.7.15.1.2	Test parameters	1642

A.6.7.15.1.3	Test requirements	1645

A.7.1	SA: RRC_IDLE state mobility	1645

A.7.1.1	Cell re-selection to NR	1645

A.7.1.1.1	Cell reselection to FR2 intra-frequency NR case	1645

A.7.1.1.1.1	Test Purpose and Environment	1645

A.7.1.1.1.2	Test Parameters	1645

A.7.1.1.1.3	Test Requirements	1649

A.7.1.1.2	Cell reselection to FR2 inter-frequency NR case	1649

A.7.1.1.2.1	Test Purpose and Environment	1649

A.7.1.1.2.2	Test Parameters	1649

A.7.1.1.2.3	Test Requirements	1653

A.7.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1653

A.7.1.1.3.1	Test Purpose and Environment	1653

A.7.1.1.3.2	Test Parameters	1653

A.7.1.1.3.3	Test Requirements	1656

A.7.1.1.4	Cell reselection to FR2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1656

A.7.1.1.4.1	Test Purpose and Environment	1656

A.7.1.1.4.2	Test Parameters	1656

A.7.1.1.4.3	Test Requirements	1659

A.7.1.1.5	Cell reselection to FR2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1659

A.7.1.1.5.1	Test Purpose and Environment	1659

A.7.1.1.5.2	Test Parameters	1659

A.7.1.1.5.3	Test Requirements	1663

A.7.1.1.6	Cell reselection to FR2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1663

A.7.1.1.6.1	Test Purpose and Environment	1663

A.7.1.1.6.2	Test Parameters	1663

A.7.1.1.6.3	Test Requirements	1665

A.7.2	SA: RRC_INACTIVE state mobility	1666

A.7.3	RRC_CONNECTED state mobility	1666

A.7.3.1	Handover	1666

A.7.3.1.1	Inter-frequency handover from FR1 to FR2; unknown target cell	1666

A.7.3.1.1.1	Test Purpose and Environment	1666

A.7.3.1.1.2	Test Parameters	1666

A.7.3.1.1.3	Test Requirements	1670

A.7.3.1.2	Intra-frequency handover from FR2 to FR2; unknown target cell	1670

A.7.3.1.2.1	Test Purpose and Environment	1670

A.7.3.1.2.2	Test Parameters	1670

A.7.3.1.2.3	Test Requirements	1673

A.7.3.1.3	Inter-frequency handover from FR2 to FR2; unknown target cell	1673

A.7.3.1.3.1	Test Purpose and Environment	1673

A.7.3.1.3.2	Test Parameters	1673

A.7.3.1.3.3 Test Requirements	1674

A.7.3.1.4	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR2	1675

A.7.3.1.4.1	Test Purpose and Environment	1675

A.7.3.1.4.2	Test Parameters	1675

A.7.3.1.4.3 Test Requirements	1682

A.7.3.1.5	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR2	1682

A.7.3.1.5.1	Test Purpose and Environment	1682

A.7.3.1.5.2	Test Parameters	1682

A.7.3.1.5.3 Test Requirements	1689

A.7.3.2	RRC Connection Mobility Control	1689

A.7.3.2.1	SA: RRC Re-establishment	1689

A.7.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	1689

A.7.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	1692

A.7.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	1695

A.7.3.2.1.3.1	Test Purpose and Environment	1695

A.7.3.2.1.3.2	Test Requirements	1697

A.7.3.2.2	Random Access	1698

A.7.3.2.2.1	4-step RA type contention based random access test in FR2 for NR Standalone	1698

A.7.3.2.2.2	4-step RA type non-contention based random access test in FR2 for NR Standalone	1702

A.7.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	1707

A.7.3.2.2.4	2-step RA type non-contention based random access test in FR2 for NR Standalone	1710

A.7.3.2.3	SA: RRC Connection Release with Redirection	1713

A.7.3.2.3.1	Redirection from NR in FR2 to NR in FR2	1713

A.7.3.3 Conditional Handover	1717

A.7.3.3.1	Intra-frequency conditional handover from FR2 to FR2	1717

A.7.3.3.1.1	Test Purpose and Environment	1717

A.7.3.3.1.2	Test Parameters	1717

A.7.3.3.1.2.3	Test Requirements	1720

A.7.3.3.2	Inter-frequency handover from FR2 to FR2; unknown target cell	1720

A.7.3.3.2.1	Test Purpose and Environment	1720

A.7.3.3.2.2	Test Parameters	1720

A.7.3.3.2.3 Test Requirements	1722

A.7.4	Timing	1722

A.7.4.1	UE transmit timing	1722

A.7.4.1.1	NR UE Transmit Timing Test for FR2	1722

A.7.4.1.1.1	Test Purpose and environment	1722

A.7.4.1.1.2	Test requirements	1725

A.7.4.2	UE timer accuracy	1725

A.7.4.3	Timing advance	1725

A.7.4.3.1	SA FR2 timing advance adjustment accuracy	1725

A.7.4.3.1.1	Test Purpose and Environment	1725

A.7.4.3.1.2	Test Parameters	1725

A.7.4.3.1.3 Test Requirements	1729

A.7.5	Signaling characteristics	1729

A.7.5.1	Radio link Monitoring	1729

A.7.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	1729

A.7.5.1.1.1	Test Purpose and Environment	1729

A.7.5.1.1.2	Test Requirements	1732

A.7.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	1733

A.7.5.1.2.1	Test Purpose and Environment	1733

A.7.5.1.2.2	Test Requirements	1737

A.7.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	1738

A.7.5.1.3.1	Test Purpose and Environment	1738

A.7.5.1.3.2	Test Requirements	1741

A.7.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	1741

A.7.5.1.4.1	Test Purpose and Environment	1741

A.7.5.1.4.2	Test Requirements	1745

A.7.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	1745

A.7.5.1.5.1	Test Purpose and Environment	1745

A.7.5.1.5.2	Test Requirements	1749

A.7.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	1749

A.7.5.1.6.1	Test Purpose and Environment	1749

A.7.5.1.6.2	Test Requirements	1753

A.7.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	1753

A.7.5.1.7.1	Test Purpose and Environment	1753

A.7.5.1.7.2	Test Requirements	1756

A.7.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	1756

A.7.5.1.8.1	Test Purpose and Environment	1756

A.7.5.1.8.2	Test Requirements	1761

A.7.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	1761

A.7.5.1.9.1	Test Purpose and Environment	1761

A.7.5.1.9.2	Test Requirements	1764

A.7.5.2	Interruption	1764

A.7.5.2.1	Interruptions during measurements on deactivated NR SCC in FR2	1764

A.7.5.2.2	SA interruptions at NR SRS carrier-based switching	1768

A.7.5.2.2.1	Test Purpose and Environment	1768

A.7.5.2.2.2	Test Parameters	1768

A.7.5.2.2.3	Test Requirements	1770

A.7.5.3	SCell Activation and Deactivation Delay	1770

A.7.5.3.1	SCell Activation and deactivation for SCell in FR2 intra-band in non-DRX	1770

A.7.5.3.1.1	Test Purpose and Environment	1770

A.7.5.3.1.2	Test Requirements	1772

A.7.5.3.2	SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2	1772

A.7.5.3.2.1	Test Purpose and Environment	1772

A.7.5.3.2.2	Test Requirements	1776

A.7.5.3.3	SCell Activation and deactivation for SCell in FR2 inter-band in non-DRX	1777

A.7.5.3.3.1	Test Purpose and Environment	1777

A.7.5.3.3.2	Test Requirements	1780

A.7.5.3.4	Direct SCell activation at SCell addition of known SCell in FR2	1781

A.7.5.3.4.1	Test Purpose and Environment	1781

A.7.5.3.4.2	Test Requirements	1784

A.7.5.3.5	Direct SCell activation at handover with known SCell in FR2	1785

A.7.5.3.5.1	Test Purpose and Environment	1785

A.7.5.3.5.2	Test Requirements	1787

A.7.5.4	Void	1788

A.7.5.5	Beam Failure Detection and Link recovery procedures	1788

A.7.5.5.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	1788

A.7.5.5.1.1	Test Purpose and Environment	1788

A.7.5.5.1.2	Test Requirements	1793

A.7.5.5.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	1793

A.7.5.5.2.1	Test Purpose and Environment	1793

A.7.5.5.2.2	Test Requirements	1798

A.7.5.5.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	1798

A.7.5.5.3.1	Test Purpose and Environment	1798

A.7.5.5.3.2	Test Requirements	1804

A.7.5.5.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	1804

A.7.5.5.4.1	Test Purpose and Environment	1804

A.7.5.5.4.2	Test Requirements	1810

A.7.5.5.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	1810

A.7.5.5.5.1	Test Purpose and Environment	1810

A.7.5.5.5.2	Test Requirements	1815

A.7.5.5.6	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	1816

A.7.5.5.6.1	Test Purpose and Environment	1816

A.7.5.5.6.2	Test Requirements	1820

A.7.5.5.7	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	1820

A.7.5.5.7.1	Test Purpose and Environment	1820

A.7.5.5.7.2	Test Requirements	1825

A.7.5.6	Active BWP switch	1825

A.7.5.6.1	DCI-based and Timer-based Active BWP Switch	1825

A.7.5.6.1.1	NR FR2- NR FR2 DL active BWP switch of SCell with non-DRX in SA	1825

A.7.5.6.1.2	NR FR1- NR FR2 DL active BWP switch of SCell with non-DRX in SA A.7.5.6.1.2.1 Test Purpose and Environment	1830

A.7.5.6.1.3	NR FR2 DL active BWP switch with non-DRX in SA	1835

A.7.5.6.1.3.1	Test Purpose and Environment	1835

A.7.5.6.1.3.2	Test Requirements	1838

A.7.5.6.2	RRC-based Active BWP Switch	1838

A.7.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1842

A.7.5.6.3.1	Active BWP switch on multiple SCells with non-DRX in SA	1842

A.7.5.6.4	SCell dormancy switch	1845

A.7.5.6.4.1	NR FR2 PCell SCell dormancy switch of single FR2 SCell inside active time	1845

A.7.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR2 SCells outside active time	1850

A.7.5.6.4.2.2	 Test Requirements	1855

A.7.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1855

A.7.5.6.5.1	Active BWP switch on multiple SCells with non-DRX in SA	1855

A.7.5.7	PSCell addition and release delay	1860

A.7.5.7.1	Addition and Release Delay of known NR PSCell	1860

A.7.5.7.1.1	Test Purpose and Environment	1860

A.7.5.7.2	Addition and Release Delay of unknown NR PSCell	1863

A.7.5.7.2.1	Test Purpose and Environment	1863

A.7.5.7.2.2	Test Requirements	1866

A.7.5.8	Active TCI state switch delay	1866

A.7.5.8.1	MAC-CE based active TCI state switch	1866

A.7.5.8.2	RRC based active TCI state switch	1869

A.7.5.9	Uplink spatial relation switch delay	1872

A.7.5.9.1	MAC-CE based Spatial Relation switch	1872

A.7.5.9.1.1	 NR PCell FR2 spatial relation associated with known DL-RS	1872

A.7.5.9.1.1.2	Test Requirements	1875

A.7.5.9.2	RRC based spatial relation switch	1875

A.7.5.9.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	1875

A.7.5.9.2.1.1	Test Purpose and Environment	1875

A.7.5.9.2.1.2	Test Requirements	1878

A.7.5.10	UE specific CBW change	1878

A.7.5.10.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	1878

A.7.5.10.1.1	Test Purpose and Environment	1878

A.7.5.10.1.2	Test Requirements	1881

A.7.6	Measurement procedure	1882

A.7.6.1	Intra-frequency Measurements	1882

A.7.6.1.1	SA event triggered reporting test without gap under non-DRX	1882

A.7.6.1.1.1	Test purpose and Environment	1882

A.7.6.1.1.2	Test Requirements	1885

A.7.6.1.2	SA event triggered reporting test without gap under DRX	1885

A.7.6.1.2.1	Test purpose and Environment	1885

A.7.6.1.2.2	Test Requirements	1888

A.7.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	1889

A.7.6.1.3.1	Test purpose and Environment	1889

A.7.6.1.3.2	Test Requirements	1892

A.7.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	1892

A.7.6.1.4.1	Test purpose and Environment	1892

A.7.6.1.4.2	Test Requirements	1895

A.7.6.2	Inter-frequency Measurements	1896

A.7.6.2.1	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2)	1896

A.7.6.2.1.1	Test Purpose and Environment	1896

A.7.6.2.1.2	Test Requirements	1899

A.7.6.2.2	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is used (PCell in FR2)	1899

A.7.6.2.2.1	Test Purpose and Environment	1899

A.7.6.2.2.2	Test Requirements	1903

A.7.6.2.3	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is not used (PCell in FR2)	1903

A.7.6.2.3.1	Test Purpose and Environment	1903

A.7.6.2.3.2	Test Requirements	1907

A.7.6.2.4	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is used (PCell in FR2)	1907

A.7.6.2.4.1	Test Purpose and Environment	1907

A.7.6.2.4.2	Test Requirements	1911

A.7.6.2.5	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1)	1911

A.7.6.2.5.1	Test Purpose and Environment	1911

A.7.6.2.5.2	Test Requirements	1915

A.7.6.2.6	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (PCell in FR1)	1916

A.7.6.2.6.1	Test Purpose and Environment	1916

A.7.6.2.6.2	Test Requirements	1920

A.7.6.2.7	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1)	1921

A.7.6.2.7.1	Test Purpose and Environment	1921

A.7.6.2.7.2	Test Requirements	1925

A.7.6.2.8	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1)	1926

A.7.6.2.8.1	Test Purpose and Environment	1926

A.7.6.2.8.2	Test Requirements	1930

A.7.6.2.9	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2) (rel16 additional mandatory gap pattern 17)	1931

A.7.6.2.9.1	Test Purpose and Environment	1931

A.7.6.2.9.2	Test Requirements	1934

A.7.6.2.10	SA event triggered reporting test without gap under non-DRX	1934

A.7.6.2.10.1	Test Purpose and Environment	1934

A.7.6.2.10.2	Test Requirements	1936

A.7.6.2.11	SA event triggered reporting test without gap under DRX	1936

A.7.6.2.11.1	Test Purpose and Environment	1936

A.7.6.2.11.2	Test Requirements	1939

A.7.6.3	L1-RSRP measurement for beam reporting	1940

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1940

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1940

A.7.6.3.1.1	Test Purpose and Environment	1940

A.7.6.3.1.2	Test parameters	1940

A.7.6.3.1.3	Test Requirements	1942

A.7.6.3.2	SSB based L1-RSRP measurement when DRX is used	1942

A.7.6.3.2.1	Test Purpose and Environment	1942

A.7.6.3.2.2	Test parameters	1943

A.7.6.3.2.3	Test Requirements	1945

A.7.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	1945

A.7.6.3.3.1	Test Purpose and Environment	1945

A.7.6.3.3.2	Test parameters	1946

A.7.6.3.3.3	Test Requirements	1948

A.7.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	1949

A.7.6.3.4.1	Test Purpose and Environment	1949

A.7.6.3.4.2	Test parameters	1949

A.7.6.3.3.3	Test Requirements	1951

A.7.6.4	CLI measurements	1952

A.7.6.4.1	SRS-RSRP measurement with non-DRX	1952

A.7.6.4.1.1	Test Purpose and Environment	1952

A.7.6.4.1.2	Test Parameters	1952

A.7.6.4.1.3	Test Requirements	1954

A.7.6.4.2	CLI-RSSI measurement with non-DRX	1954

A.7.6.4.2.1	Test Purpose and Environment	1954

A.7.6.4.2.2	Test Parameters	1955

A.7.6.4.2.3	Test Requirements	1956

A.7.6.5	NR Measurements with autonomous gaps	1957

A.7.6.5.1	SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2)	1957

A.7.6.5.1.1	Test Purpose and Environment	1957

A.7.6.5.1.2	Test Requirements	1960

A.7.6.6	L1-SINR measurement for beam reporting	1960

A.7.6.6.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is not used	1960

A.7.6.6.1.1	Test Purpose and Environment	1960

A.7.6.6.1.2	Test parameters	1960

A.7.6.6.1.3	Test Requirements	1962

A.7.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	1962

A.7.6.6.2.1	Test Purpose and Environment	1962

A.7.6.6.2.2	Test parameters	1963

A.7.6.6.2.3	Test Requirements	1965

A.7.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	1966

A.7.6.6.3.1	Test Purpose and Environment	1966

A.7.6.6.3.2	Test parameters	1966

A.7.6.6.3.3	Test Requirements	1968

A.7.6.7	CSI-RS based intra-frequency Measurements	1968

A.7.6.7.1	SA event triggered reporting test without gap under DRX for CSI-RS based intra-frequency measurement	1968

A.7.6.7.1.1	Test purpose and Environment	1968

A.7.6.7.1.2	Test Requirements	1971

A.7.6.8	CSI-RS based inter-frequency Measurements	1971

A.7.6.8.1	SA event triggered reporting tests for FR2 CSI-RS based measurement when non-DRX is used (PCell in FR2)	1971

A.7.6.8.1.1	Test Purpose and Environment	1971

A.7.6.2.2.2	Test Requirements	1975

A.7.6.9	RSTD measurements	1975

A.7.6.9.1	 NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA	1975

A.7.6.9.1.1	Test Purpose and Environment	1975

A.7.6.9.1.2	Test Requirements	1981

A.7.6.9.2	 NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR2 SA	1981

A.7.6.9.2.1	Test Purpose and Environment	1981

A.7.6.9.2.2	Test Requirements	1988

A.7.6.10 PRS-RSRP measurements	1988

A.7.6.10.1 PRS-RSRP reporting delay test case for single positioning frequency layer	1988

A.7.6.10.1.1	Test Purpose and Environment	1988

A.7.6.10.1.2	Test Requirements	1992

A.7.6.10.2 PRS-RSRP reporting delay test case for dual positioning frequency layer	1992

A.7.6.10.2.1	Test Purpose and Environment	1992

A.7.6.10.2.2	Test Requirements	1996

A.7.6.11	UE Rx-Tx time difference measurements	1996

A.7.6.11.1 UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	1996

A.7.6.11.1.1	Test purpose and environment	1996

A.7.6.11.1.2	Test requirements	1999

A.7.6.11.2 UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR2 SA	1999

A.7.6.11.2.1	Test purpose and environment	1999

A.7.6.11.2.2	Test requirements	2002

A.7.7	Measurement Performance requirements	2002

A.7.7.1	SS-RSRP	2002

A.7.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2002

A.7.7.1.1.1	Test Purpose and Environment	2002

A.7.7.1.1.2	Test parameters	2003

A.7.7.1.1.3	Test Requirements	2007

A.7.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2008

A.7.7.1.2.1	Test Purpose and Environment	2008

A.7.7.1.2.2	Test parameters	2008

A.7.7.1.2.3	Test Requirements	2012

A.7.7.1.3	SA inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	2013

A.7.7.1.3.1	Test Purpose and Environment	2013

A.7.7.1.3.2	Test parameters	2013

A.7.7.1.3.3	Test Requirements	2017

A.7.7.2	SS-RSRQ	2017

A.7.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	2017

A.7.7.2.1.1	Test Purpose and Environment	2017

A.7.7.2.1.2	Test Parameters	2017

A.7.7.2.1.3	Test Requirements	2019

A.7.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2019

A.7.7.2.2.1	Test Purpose and Environment	2019

A.7.7.2.2.2	Test Parameters	2019

A.7.7.2.2.3	Test Requirements	2021

A.7.7.3	SS-SINR	2021

A.7.7.3.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2021

A.7.7.3.1.1	Test Purpose and Environment	2021

A.7.7.3.1.2	Test Parameters	2021

A.7.7.3.1.3	Test Requirements	2024

A.7.7.3.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2024

A.7.7.3.2.1	Test Purpose and Environment	2024

A.7.7.3.2.2	Test Parameters	2024

A.7.7.3.2.3	Test Requirements	2026

A.7.7.4	L1-RSRP measurement for beam reporting	2026

A.7.7.4.1	SSB based L1-RSRP measurement	2026

A.7.7.4.1.1	Test Purpose and Environment	2026

A.7.7.4.1.2	Test parameters	2027

A.7.7.4.1.3	Test Requirements	2029

A.7.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	2030

A.7.7.4.2.1	Test Purpose and Environment	2030

A.7.7.4.2.2	Test parameters	2030

A.7.7.4.2.3	Test Requirements	2032

A.7.7.5	CLI measurements	2033

A.7.7.5.1	SA SRS-RSRP measurement accuracy with FR2 serving cell	2033

A.7.7.5.1.1	Test Purpose and Environment	2033

A.7.7.5.1.2	Test parameters	2033

A.7.7.5.1.3	Test Requirements	2036

A.7.7.5.2	SA CLI-RSSI measurement accuracy with FR2 serving cell	2037

A.7.7.5.2.1	Test Purpose and Environment	2037

A.7.7.5.2.2	Test parameters	2037

A.7.7.5.2.3	Test Requirements	2039

A.7.7.6	L1-SINR measurement for beam reporting	2040

A.7.7.6.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	2040

A.7.7.6.1.1	Test Purpose and Environment	2040

A.7.7.6.1.2	Test parameters	2040

A.7.7.6.1.3	Test Requirements	2042

A.7.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	2042

A.7.7.6.2.1	Test Purpose and Environment	2042

A.7.7.6.2.2	Test parameters	2043

A.7.7.6.2.3	Test Requirements	2045

A.7.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	2045

A.7.7.6.3.1	Test Purpose and Environment	2045

A.7.7.6.3.2	Test parameters	2046

A.7.7.6.3.3	Test Requirements	2048

A.7.7.7	CSI-RSRP	2048

A.7.7.7.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2048

A.7.7.7.1.1	Test Purpose and Environment	2048

A.7.7.7.1.2	Test parameters	2049

A.7.7.7.1.3	Test Requirements	2052

A.7.7.7.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2053

A.7.7.7.2.1	Test Purpose and Environment	2053

A.7.7.7.2.2	Test parameters	2053

A.7.7.7.2.3	Test Requirements	2056

A.7.7.8	CSI-RSRQ	2056

A.7.7.8.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	2056

A.7.7.8.1.1	Test Purpose and Environment	2056

A.7.7.8.1.2	Test Parameters	2057

A.7.7.8.1.3	Test Requirements	2059

A.7.7.8.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2059

A.7.7.8.2.1	Test Purpose and Environment	2059

A.7.7.8.2.2	Test Parameters	2059

A.7.7.8.2.3	Test Requirements	2061

A.7.7.9	CSI-SINR	2061

A.7.7.9.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2061

A.7.7.9.1.1	Test Purpose and Environment	2061

A.7.7.9.1.2	Test Parameters	2061

A.7.7.9.1.3	Test Requirements	2064

A.7.7.9.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2064

A.7.7.9.2.2	Test Parameters	2064

A.7.7.9.2.3	Test Requirements	2066

A.7.7.10	RSTD measurements	2066

A.7.7.10.1	RSTD measurement accuracy test case for single positioning frequency layer	2066

A.7.7.10.1.1	Test purpose and Environment	2066

A.7.7.10.1.2	Test Requirements	2069

A.7.7.10.2	RSTD measurement accuracy test case for dual positioning frequency layer	2069

A.7.7.10.2.1	Test purpose and Environment	2069

A.7.7.10.2.2	Test Requirements	2071

A.7.7.11	PRS-RSRP measurements	2071

A.7.7.11.1	SA measurement accuracy with PRS in FR2	2071

A.7.7.11.1.1	Test Purpose and Environment	2071

A.7.7.11.1.2	Test parameters	2071

A.7.7.11.1.3	Test Requirements	2074

A.7.7.12	UE Rx-Tx time difference measurements	2075

A.7.7.12.1 UE Rx-Tx time difference measurement period for single positioning frequency layer in FR2 SA	2075

A.7.7.12.1.1	Test purpose and environment	2075

A.7.7.12.1.2	Test parameters	2075

A.7.7.12.1.3	Test requirements	2077

A.8	E-UTRA standalone tests for NR RRM	2077

A.8.1	Void	2077

A.8.2	RRC_IDLE state mobility	2077

A.8.2.1	Inter-RAT NR Cell re-selection	2077

A.8.2.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	2077

A.8.2.1.1.1	Test Purpose and Environment	2077

A.8.2.1.1.2	Test Requirements	2082

A.8.2.1.2	E-UTRA Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16	2083

A.8.2.1.2.1	Test Purpose and Environment	2083

A.8.2.1.2.2	Test Requirements	2088

A.8.2.2	E-UTRA – NR Inter-RAT Early Measruement Reporting	2089

A.8.2.2.1	E-UTRA – NR Early Measruement Reporting for NR in FR1	2089

A.8.2.2.1.1	Test Purpose and Environment	2089

A.8.2.2.1.2	Test Requirements	2094

A.8.3	RRC_CONNECTED state mobility	2094

A.8.3.1	Handover	2094

A.8.3.1.1	E-UTRAN - NR handover in FR1	2094

A.8.3.1.1.1	Test Purpose and Environment	2094

A.8.3.1.1.2	Test Requirements	2099

A.8.4	Measurement procedure	2099

A.8.4.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay	2099

A.8.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay in non-DRX	2099

A.8.4.1.1.1	Test Purpose and Environment	2099

A.8.4.1.1.2	Test Requirements	2101

A.8.4.1.2	E-UTRA – NR Inter-RAT SFTD Measurement Delay in DRX	2102

A.8.4.1.2.1	Test Purpose and Environment	2102

A.8.4.1.2.2	Test Requirements	2103

A.8.4.2	E-UTRA – NR Inter-RAT Measurements	2103

A.8.4.2.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2103

A.8.4.2.1.1	Test Purpose and Environment	2103

A.8.4.2.1.2	Test Requirements	2108

A.8.4.2.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2108

A.8.4.2.2.1	Test Purpose and Environment	2108

A.8.4.2.2.2	Test Requirements	2112

A.8.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2112

A.8.4.2.3.1	Test Purpose and Environment	2112

A.8.4.2.3.2	Test Requirements	2117

A.8.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2117

A.8.4.2.4.1	Test Purpose and Environment	2117

A.8.4.2.4.2	Test Requirements	2121

A.8.4.2.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	2121

A.8.4.2.5.1	Test Purpose and Environment	2121

A.8.4.2.5.2	Test Requirements	2123

A.8.4.2.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	2124

A.8.4.2.6.1	Test Purpose and Environment	2124

A.8.4.2.6.2	Test Requirements	2126

A.8.4.2.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	2127

A.8.4.2.7.1	Test Purpose and Environment	2127

A.8.4.2.7.2	Test Requirements	2129

A.8.4.2.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	2130

A.8.4.2.8.1	Test Purpose and Environment	2130

A.8.4.2.8.2	Test Requirements	2132

A.8.4.2.9	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16	2133

A.8.4.2.9.1	Test Purpose and Environment	2133

A.8.4.2.9.2	Test Requirements	2139

A.8.5	Measurement performance	2139

A.8.5.1	SFTD accuracy	2139

A.8.5.1.1	SFTD accuracy	2139

A.8.5.1.1.1	Test Purpose	2139

A.8.5.1.1.2	Test Environment	2139

A.8.5.1.1.3	Test Requirements	2145

A.8.5.2	E-UTRA – NR Inter-RAT Measurement Performance requirements	2145

A.8.5.2.1.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	2145

A.8.5.2.1.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	2150

A.8.5.2.1.2.1	Test Purpose and Environment	2150

A.8.5.2.1.2.2	Test Parameters	2150

A.8.5.2.1.2.3	Test Requirements	2152

A.8.5.2.2	SS-RSRQ	2152

A.8.5.2.2.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	2152

A.8.5.2.2.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	2157

A.8.5.2.2.2.1	Test Purpose and Environment	2157

A.8.5.2.2.2.2	Test Parameters	2157

A.8.5.2.2.2.3	Test Requirements	2159

A.8.5.2.3	SS-SINR	2159

A.8.5.2.3.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	2159

A.8.5.2.3.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	2163

A.8.5.2.3.2.1	Test Purpose and Environment	2163

A.8.5.2.3.2.2	Test Parameters	2163

A.8.5.2.3.2.3	Test Requirements	2165

A.9	V2X Tests	2166

A.9.1	V2X Tests in FR1	2166

A.9.1.1	Test for V2X UE Transmit Timing	2166

A.9.1.1.1	Test for GNSS as Synchronization Reference Source	2166

A.9.1.1.1.1	Test Purpose and Environment	2166

A.9.1.1.1.2	Test requirements	2166

A.9.1.1.2	Test for SyncRef UE as Synchronization Reference Source	2166

A.9.1.1.2.1	Test Purpose and Environment	2166

A.9.1.1.2.2	Test requirements	2167

A.9.1.1.3	Test for FR1 NR Cell as Synchronization Reference Source	2167

A.9.1.1.3.1	Test Purpose and Environment	2167

A.9.1.1.3.2	Test requirements	2170

A.9.1.2	Test for Initiation/Cease of SLSS Transmission with V2X Sidelink Communication	2170

A.9.1.2.1	Test for FR1 NR Cell as synchronization reference source without gap under non-DRX	2170

A.9.1.2.1.1	Test Purpose and Environment	2170

A.9.1.2.1.2	Test Requirements	2174

A.9.1.2.2	Test for SyncRef UE as synchronization reference source	2174

A.9.1.2.2.1	Test Purpose and Environment	2174

A.9.1.2.2.2	Test Requirements	2175

A.9.1.3	Test for V2X Synchronization Reference Selection/Reselection	2176

A.9.1.3.1	Test for GNSS configured as the highest priority	2176

A.9.1.3.1.1	Test Purpose and Environment	2176

A.9.1.3.1.2	Test Requirements	2178

A.9.1.3.2	 Test for FR1 NR Cell configured as the highest priority	2179

A.9.1.3.2.1	Test Purpose and Environment	2179

A.9.1.3.2.2	Test Requirements	2182

A.9.1.4	Test for L1 SL-RSRP Measurement	2183

A.9.1.4.1	Test for V2X UE Autonomous Resource Selection/Reselection	2183

A.9.1.4.1.1	Test Purpose and Environment	2183

A.9.1.4.1.2	Test Requirements	2185

A.9.1.4.2	Test for V2X UE Resource Pre-emption	2185

A.9.1.4.2.1	Test Purpose and Environment	2185

A.9.1.4.2.2	Test Requirements	2189

A.9.1.4.3	Test for V2X UE Resource Re-evaluation	2189

A.9.1.4.3.1	Test Purpose and Environment	2189

A.9.1.4.3.2	Test Requirements	2194

A.9.1.5	Test for Congestion Control Measurement	2194

A.9.1.5.1	Test Purpose and Environment	2194

A.9.1.5.2	Test Requirements	2196

A.9.1.6	Test for Interruption	2196

A.9.1.6.1	Test for Interruption to WAN due to V2X Sidelink Communication	2196

A.9.1.6.1.1	Test Purpose and Environment	2196

A.9.1.6.1.2	Test Requirements	2200

A.10	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	2200

A.10.1	RRC_CONNECTED state mobility	2200

A.10.1.1	RRC connection mobility control	2200

A.10.1.1.1	Random Access	2200

A.10.1.1.1.1	4-step RA type contention-based random access for NR PSCell with CCA	2200

A.10.1.1.1.1.1	Test Purpose and Environment	2200

A.10.1.1.1.1.2	Test Requirements	2203

A.10.1.1.1.1.2.1	Random Access Preamble Transmission	2203

A.10.1.1.1.1.2.2	Random Access Response Reception	2203

A.10.1.1.1.1.2.3	No Random Access Response Reception	2204

A.10.1.1.1.1.2.4	Receiving an UL grant for msg3 retransmission	2204

A.10.1.1.1.1.2.5	 Contention Resolution Timer expiry	2204

A.10.1.1.1.2	4-step RA type non-contention based random access for NR PSCell with CCA	2205

A.10.1.1.1.2.1	Test Purpose and Environment	2205

A.10.1.1.1.2.2	Test Requirements	2208

A.10.1.1.1.2.2.1	SSB-based Random Access Preamble Transmission	2208

A.10.1.1.1.2.2.2	Random Access Response Reception	2209

A.10.1.1.1.2.2.3	No Random Access Response Reception	2209

A.10.1.1.1.3	2-step RA type contention-based random access for NR PSCell with CCA	2209

A.10.1.1.1.3.1	Test Purpose and Environment	2209

A.10.1.1.1.3.2	Test Requirements	2213

A.10.1.1.1.3.2.1	MsgA Transmission	2213

A.10.1.1.1.3.2.2	MsgB Reception	2213

A.10.1.1.1.3.2.3	No MsgB Reception	2214

A.10.1.1.1.4	2-step RA type non-contention based random access for NR PSCell with CCA	2214

A.10.1.1.1.4.1	Test Purpose and Environment	2214

A.10.1.1.1.4.2	Test Requirements	2218

A.10.1.1.1.4.2.1	MsgA Transmission	2218

A.10.1.1.1.4.2.2	MsgB Reception	2219

A.10.1.1.1.4.2.3	No MsgB Reception	2219

A.10.2	Timing	2220

A.10.2.1	UE transmit timing	2220

A.10.2.2	UE timing advance	2224

A.10.3	Signalling characteristics	2227

A.10.3.1	Radio link monitoring	2227

A.10.3.1.1	Introduction	2227

A.10.3.1.2	Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	2228

A.10.3.1.2.1	Test purpose and environment	2228

A.10.3.1.2.2	Test requirements	2232

A.10.3.1.3	Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	2232

A.10.3.1.3.1	Test purpose and environment	2232

A.10.3.1.3.2	Test requirements	2238

A.10.3.1.4	Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode	2238

A.10.3.1.4.1	Test purpose and environment	2238

A.10.3.1.4.2	Test requirements	2238

A.10.3.1.5	Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode	2238

A.10.3.1.5.1	Test purpose and environment	2238

A.10.3.1.5.2	Test requirements	2238

A.10.3.2	Interruption	2238

A.10.3.2.1	E-UTRAN – NR interruptions during SCell operations with CCA	2238

A.10.3.2.1.1	Test Purpose and Environment	2238

A.10.3.2.1.2	Test Requirements	2243

A.10.3.3	SCell activation and deactivation delay	2243

A.10.3.4	Beam failure detection and link recovery procedures	2250

A.10.3.5	Active BWP switching	2263

A.10.3.5.1	UL active BWP switch delay with consistent UL LBT failure on PSCell subject to UL CCA in EN-DC	2263

A.10.3.5.1.2	Test Requirements	2268

A.10.3.5.2	DCI-based and Timer-based Active BWP Switch	2269

A.10.3.5.2.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	2269

A.10.3.5.2.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	2272

A.10.3.5.3	RRC-based Active BWP Switch	2276

A.10.3.6	PSCell addition and release delay	2280

A.10.3.6.1	Addition and Release Delay of known NR PSCell on the carrier under CCA	2280

A.10.3.6.1.1	Test purpose and environment	2280

A.10.3.6.1.2	Test Requirements	2284

A.10.3.7	Active TCI state switching delay	2285

A.10.4	Measurement procedure	2285

A.10.4.1	Intra-frequency measurements	2285

A.10.4.1.1	Event-triggered reporting tests on PSCC without gaps under non-DRX	2285

A.10.4.1.1.1	Test purpose and environment	2285

A.10.4.1.1.2	Test parameters	2285

A.10.4.1.1.3	Test Requirements	2285

A.10.4.1.2	Event-triggered reporting tests on PSCC without gaps under DRX	2286

A.10.4.1.2.1	Test purpose and environment	2286

A.10.4.1.2.2	Test parameters	2286

A.10.4.1.2.3	Test Requirements	2286

A.10.4.1.3	Event-triggered reporting tests on PSCC with per-UE gaps under non-DRX	2286

A.10.4.1.3.1	Test purpose and environment	2286

A.10.4.1.3.2	Test parameters	2286

A.10.4.1.3.3	Test Requirements	2286

A.10.4.1.4	Event-triggered reporting tests on PSCC with per-UE gaps under DRX	2287

A.10.4.1.4.1	Test purpose and environment	2287

A.10.4.1.4.2	Test parameters	2287

A.10.4.1.4.3	Test Requirements	2287

A.10.4.1.5	Event-triggered reporting tests on SCC without gaps under non-DRX	2287

A.10.4.1.5.1	Test purpose and environment	2287

A.10.4.1.5.2	Test parameters	2287

A.10.4.1.5.3	Test Requirements	2287

A.10.4.1.6	Event-triggered reporting tests on SCC without gaps under DRX	2288

A.10.4.1.6.1	Test purpose and environment	2288

A.10.4.1.6.2	Test parameters	2288

A.10.4.1.6.3	Test Requirements	2288

A.10.4.1.7	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	2288

A.10.4.1.7.1	Test purpose and environment	2288

A.10.4.1.7.2	Test parameters	2288

A.10.4.1.7.3	Test Requirements	2288

A.10.4.1.8	Event-triggered reporting tests on SCC with per-UE gaps under DRX	2289

A.10.4.1.8.1	Test purpose and environment	2289

A.10.4.1.8.2	Test parameters	2289

A.10.4.1.8.3	Test Requirements	2289

A.10.4.1.9	RSSI measurement reporting on PSCC	2289

A.10.4.1.9.1	Test purpose and environment	2289

A.10.4.1.9.2	Test parameters	2289

A.10.4.1.10	Channel occupancy measurement reporting on PSCC	2290

A.10.4.1.10.1	Test purpose and environment	2290

A.10.4.1.10.2	Test parameters	2290

A.10.4.1.11	RSSI measurement reporting on SCC	2290

A.10.4.1.11.1	Test purpose and environment	2290

A.10.4.1.11.2	Test parameters	2290

A.10.4.1.12	Channel occupancy measurement reporting on SCC	2291

A.10.4.1.12.1	Test purpose and environment	2291

A.10.4.1.12.2	Test parameters	2291

A.10.4.2	Inter-frequency measurements	2291

A.10.4.2.1	RSSI measurement reporting	2291

A.10.4.2.1.1	Test purpose and environment	2291

A.10.4.2.1.2	Test parameters	2291

A.10.4.2.2	Channel occupancy measurement reporting	2292

A.10.4.2.2.1	Test purpose and environment	2292

A.10.4.2.2.2	Test parameters	2292

A.10.4.2.3	EN-DC event triggered reporting tests for FR1 with CCA cell without SSB time index detection when DRX is not used	2292

A.10.4.2.3.1	Test Purpose and Environment	2292

A.10.4.2.3.2	Test Requirements	2297

A.10.4.2.4	EN-DC event triggered reporting tests for FR1 cell with CCA without SSB time index detection when DRX is used	2298

A.10.4.2.4.1	Test Purpose and Environment	2298

A.10.4.2.4.2	Test Requirements	2302

A.10.4.2.5	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is not used	2303

A.10.4.2.5.1	Test Purpose and Environment	2303

A.10.4.2.5.2	Test Requirements	2307

A.10.4.2.6	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is used	2308

A.10.4.2.6.1	Test Purpose and Environment	2308

A.10.4.2.6.2	Test Requirements	2312

A.10.4.2.7	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	2313

A.10.4.2.7.1	Test Purpose and Environment	2313

A.10.4.2.7.2	Test Requirements	2319

A.10.4.2.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	2319

A.10.4.2.8.1	Test Purpose and Environment	2319

A.10.4.2.8.2	Test Requirements	2325

A.10.4.2.9	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	2326

A.10.4.2.9.1	Test Purpose and Environment	2326

A.10.4.2.9.2	Test Requirements	2331

A.10.4.2.10	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	2331

A.10.4.2.10.1	Test Purpose and Environment	2331

A.10.4.2.10.2	Test Requirements	2337

A.10.4.3	L1-RSRP measurements for beam reporting	2338

A.10.4.3.1	SSB based L1-RSRP measurement on PSCC when DRX is not used	2338

A.10.4.3.1.1	Test Purpose and Environment	2338

A.10.4.3.1.2	Test parameters	2338

A.10.4.3.1.3	Test Requirements	2340

A.10.4.3.2	SSB based L1-RSRP measurement on PSCC when DRX is used	2341

A.10.4.3.2.1	Test Purpose and Environment	2341

A.10.4.3.2.2	Test parameters	2341

A.10.4.3.2.3	Test Requirements	2343

A.10.4.3.3	SSB based L1-RSRP measurement on SCC when DRX is not used	2344

A.10.4.3.3.1	Test Purpose and Environment	2344

A.10.4.3.3.2	Test parameters	2344

A.10.4.3.3.3	Test Requirements	2347

A.10.4.3.4	SSB based L1-RSRP measurement on SCC when DRX is used	2348

A.10.4.3.4.1	Test Purpose and Environment	2348

A.10.4.3.4.2	Test parameters	2348

A.10.4.3.4.3	Test Requirements	2351

A.10.4.4	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	2352

A.10.4.4.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2352

A.10.4.4.1.1	Test Purpose and Environment	2352

A.10.4.4.1.2	Test Requirements	2356

A.10.4.4.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2357

A.10.4.4.2.1	Test Purpose and Environment	2357

A.10.4.4.2.2	Test Requirements	2362

A.10.4.4.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2363

A.10.4.4.3.1	Test Purpose and Environment	2363

A.10.4.4.3.2	Test Requirements	2367

A.10.4.4.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2368

A.10.4.4.4.1	Test Purpose and Environment	2368

A.10.4.4.4.2	Test Requirements	2373

A.10.5	Measurement performance	2374

A.10.5.1	SS-RSRP	2374

A.10.5.1.1	Intra-frequency measurement accuracy on a CCA serving cell	2374

A.10.5.1.1.1	Test Purpose and Environment	2374

A.10.5.1.1.2	Test parameters	2374

A.10.5.1.1.3	Test Requirements	2377

A.10.5.1.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	2377

A.10.5.1.2.1	Test Purpose and Environment	2377

A.10.5.1.2.2	Test parameters	2377

A.10.5.1.2.3	Test Requirements	2380

A.10.5.2	SS-RSRQ	2380

A.10.5.2.1	Intra-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	2380

A.10.5.2.1.1	Test Purpose and Environment	2380

A.10.5.2.1.2	Test Parameters	2380

A.10.5.2.1.3	Test Requirements	2383

A.10.5.2.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	2383

A.10.5.2.2.1	Test Purpose and Environment	2383

A.10.5.2.2.2	Test Parameters	2383

A.10.5.2.2.3	Test Requirements	2386

A.10.5.3	SS-SINR	2386

A.10.5.4	L1-RSRP measurement for beam reporting with CCA serving cell	2386

A.10.5.4.1	SSB based L1-RSRP measurement	2386

A.10.5.4.1.1	Test Purpose and Environment	2386

A.10.5.4.1.2	Test parameters	2386

A.10.5.4.1.3	Test Requirements	2389

A.10.5.5	RSSI	2389

A.10.5.5.1 	RSSI measurement accuracy on PSCC with CCA	2389

A.10.5.5.1.1	Test Purpose and Environment	2389

A.10.5.5.1.2	Test parameters	2389

A.10.5.5.1.3	Test Requirements	2392

A.10.5.5.2 	RSSI measurement accuracy on SCC with CCA	2392

A.10.5.5.2.1	Test Purpose and Environment	2392

A.10.5.5.2.2	Test parameters	2392

A.10.5.5.2.3	Test Requirements	2395

A.10.5.5.3 	Inter-frequency RSSI measurement accuracy on a carrier with CCA	2395

A.10.5.5.3.1	Test Purpose and Environment	2395

A.10.5.5.3.2	Test parameters	2395

A.10.5.5.3.3	Test Requirements	2399

A.10.5.6	Channel occupancy	2399

A.10.5.6.1 	Channel occupancy measurement accuracy on PSCC with CCA	2399

A.10.5.6.1.1	Test Purpose and Environment	2399

A.10.5.6.1.2	Test parameters	2399

A.10.5.6.1.3	Test Requirements	2403

A.10.5.6.2 	Channel occupancy measurement accuracy on SCC with CCA	2403

A.10.5.6.2.1	Test Purpose and Environment	2403

A.10.5.6.2.2	Test parameters	2403

A.10.5.6.2.3	Test Requirements	2406

A.10.5.6.3 	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	2406

A.10.5.6.3.1	Test Purpose and Environment	2406

A.10.5.6.3.2	Test parameters	2406

A.10.5.6.3.3	Test Requirements	2410

A.11	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	2410

A.11.1	RRC_IDLE state mobility	2410

A.11.1.1	Cell re-selection with both source and target NR carrier frequencies under CCA	2410

A.11.1.1.1	Cell reselection to FR1 intra-frequency NR cells when subject to CCA on the serving and target cell	2410

A.11.1.1.1.1	Test Purpose and Environment	2410

A.11.1.1.1.2	Test Parameters	2411

A.11.1.1.1.3	Test Requirements	2414

A.11.1.1.2	Cell reselection to FR1 inter-frequency NR case when subject to CCA on the serving and target cell	2414

A.11.1.1.2.1	Test Purpose and Environment	2414

A.11.1.1.2.2	Test Parameters	2414

A.11.1.1.2.3	Test Requirements	2418

A.11.1.2	Cell re-selection to NR with source NR carrier frequency under CCA	2418

A.11.1.2.1	Cell reselection to FR1 inter-frequency NR case when serving cell is subject to CCA	2418

A.11.1.2.1.1	Test Purpose and Environment	2418

A.11.1.2.1.2	Test Parameters	2419

A.11.1.2.1.3	Test Requirements	2425

A.11.1.3	Cell re-selection from NR carrier with target NR carrier frequency under CCA	2426

A.11.1.3.1	Cell reselection to FR1 inter-frequency NR case when target cell is subject to CCA	2426

A.11.1.3.1.1	Test Purpose and Environment	2426

A.11.1.3.1.2	Test Parameters	2426

A.11.1.3.1.3	Test Requirements	2431

A.11.1.4	Inter-RAT cell re-selection to E-UTRAN with source NR carrier frequency under CCA	2432

A.11.1.4.1	Cell reselection to higher priority E-UTRAN when serving cell is subject to CCA	2432

A.11.1.4.1.1	Test Purpose and Environment	2432

A.11.1.4.1.2	Test Parameters	2432

A.11.1.4.1.3	Test Requirements	2435

A.11.1.4.2	Cell reselection to lower priority E-UTRAN when serving cell is subject to CCA	2436

A.11.1.4.2.1	Test Purpose and Environment	2436

A.11.1.4.2.2	Test Requirements	2438

A.11.2	RRC_CONNECTED state mobility	2439

A.11.2.1	Handover	2439

A.11.2.1.1	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; known target cell	2439

A.11.2.1.1.1	Test Purpose and Environment	2439

A.11.2.1.1.2	Test Parameters	2439

A.11.2.1.1.3 Test Requirements	2442

A.11.2.1.2	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	2442

A.11.2.1.2.1	Test Purpose and Environment	2442

A.11.2.1.2.2	Test Parameters	2442

A.11.2.1.2.3	Test Requirements	2445

A.11.2.1.3	Inter-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	2445

A.11.2.1.3.1	Test Purpose and Environment	2445

A.11.2.1.3.2	Test Parameters	2445

A.11.2.1.3.3 Test Requirements	2448

A.11.2.1.4	Inter-frequency handover from FR1 carrier under CCA to FR1; known target cell	2448

A.11.2.1.4.1	Test Purpose and Environment	2448

A.11.2.1.4.2	Test Parameters	2448

A.11.2.1.4.3 Test Requirements	2452

A.11.2.1.5	Inter-frequency handover from FR1 carrier under CCA to FR1; unknown target cell	2453

A.11.2.1.5.1	Test Purpose and Environment	2453

A.11.2.1.5.2	Test Parameters	2453

A.11.2.1.5.3 Test Requirements	2456

A.11.2.1.6	Inter-frequency handover from FR1 to FR1 carrier under CCA; unknown target cell	2457

A.11.2.1.6.1	Test Purpose and Environment	2457

A.11.2.1.6.2	Test Parameters	2457

A.11.2.1.6.3 Test Requirements	2460

A.11.2.1.7	 SA NR FR1 carrier under CCA - E-UTRAN handover with known target cell	2461

A.11.2.1.7.1	Test Purpose and Environment	2461

A.11.2.1.7.2	Test Requirements	2467

A.11.2.1.8	SA NR FR1 carrier under CCA - E-UTRAN handover with unknown target cell	2467

A.11.2.1.8.1	Test Purpose and Environment	2467

A.11.2.1.8.2	Test Requirements	2472

A.11.2.2	RRC connection mobility control	2472

A.11.2.2.1	RRC re-establishment	2472

A.11.2.2.1.1	Intra-frequency RRC Re-establishment with CCA in FR1	2472

A.11.2.2.1.2	Inter-frequency RRC Re-establishment with CCA in FR1	2475

A.11.2.2.1.4	Inter-frequency RRC Re-establishment from NR FR1 carrier without CCA to NR FR1 carrier under CCA	2481

A.11.2.2.2	Random Access	2486

A.11.2.2.2.1	4-step RA type contention-based random access for NR PCell with CCA	2486

A.11.2.2.2.1.1	Test Purpose and Environment	2486

A.11.2.2.2.1.2	Test Requirements	2489

A.11.2.2.2.1.2.1	Random Access Preamble Transmission	2489

A.11.2.2.2.1.2.2	Random Access Response Reception	2489

A.11.2.2.2.1.2.3	No Random Access Response Reception	2490

A.11.2.2.2.1.2.4	Receiving an UL grant for msg3 retransmission	2490

A.11.2.2.2.1.2.5	Reception of an Incorrect Message over Temporary C-RNTI	2490

A.11.2.2.2.1.2.6	Reception of a Correct Message over Temporary C-RNTI	2491

A.11.2.2.2.1.2.7	Contention Resolution Timer expiry	2491

A.11.2.2.2.2	4-step RA type non-contention based random access for NR PSCell with CCA	2491

A.11.2.2.2.2.1	Test Purpose and Environment	2491

A.11.2.2.2.2.2	Test Requirements	2494

A.11.2.2.2.2.2.1	SSB-based Random Access Preamble Transmission	2494

A.11.2.2.2.2.2.2	Random Access Response Reception	2495

A.11.2.2.2.2.2.3	No Random Access Response Reception	2495

A.11.2.2.2.3	2-step RA type contention-based random access for NR PCell with CCA	2495

A.11.2.2.2.3.1	Test Purpose and Environment	2495

A.11.2.2.2.3.2	Test Requirements	2499

A.11.2.2.2.3.2.1	MsgA Transmission	2499

A.11.2.2.2.3.2.2	MsgB Reception	2499

A.11.2.2.2.3.2.3	No MsgB Reception	2500

A.11.2.2.2.4	2-step RA type non-contention-based random access for NR PCell with CCA	2500

A.11.2.2.2.4.1	Test Purpose and Environment	2500

A.11.2.2.2.4.2	Test Requirements	2504

A.11.2.2.2.4.2.1	MsgA Transmission	2504

A.11.2.2.2.4.2.2	MsgB Reception	2505

A.11.2.2.2.4.2.3	No MsgB Reception	2505

A.11.2.2.3	RRC connection release with redirection	2506

A.11.2.2.3.1	Redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA	2506

A.11.3	Timing	2514

A.11.3.1	UE transmit timing	2514

A.11.3.2	UE timing advance	2518

A.11.3.2.1	UE Timing Advance Adjustment Accuracy with PCell under DL CCA	2518

A.11.3.2.1.1	Test Purpose and Environment	2518

A.11.3.2.1.2	Test Parameters	2518

A.11.3.2.1.3	Test Requirements	2522

A.11.4	Signalling characteristics	2522

A.11.4.1	Radio link monitoring	2522

A.11.4.1.1	Introduction	2522

A.11.4.1.2	Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	2523

A.11.4.1.2.1	Test purpose and environment	2523

A.11.4.1.2.2	Test requirements	2527

A.11.4.1.3	Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	2527

A.11.4.1.3.1	Test purpose and environment	2527

A.11.4.1.3.2	Test requirements	2533

A.11.4.1.4	Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode	2533

A.11.4.1.4.1	Test purpose and environment	2533

A.11.4.1.4.2	Test requirements	2533

A.11.4.1.5	Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode	2533

A.11.4.1.5.1	Test purpose and environment	2533

A.11.4.1.5.2	Test requirements	2533

A.11.4.2	Interruption	2534

A.11.4.2.1	NR interruptions during Scell operations with CCA on PCell and SCell	2534

A.11.4.2.1.1 Test Purpose and Environment	2534

A.11.4.2.1.2	Test Requirements	2538

A.11.4.3	SCell activation and deactivation delay	2538

A.11.4.4	Beam failure detection and link recovery procedures	2545

A.11.4.4.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	2545

A.11.4.4.1.1	Test Purpose and Environment	2545

A.11.4.4.1.2	Test Requirements	2551

A.11.4.4.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	2551

A.11.4.4.2.1	Test Purpose and Environment	2551

A.11.4.4.2.2	Test Requirements	2558

A.11.4.5	Active BWP switching	2558

A.11.4.5.1	UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA	2558

A.11.4.5.1.2	Test Requirements	2562

A.11.4.6	Active TCI state switching delay	2572

A.11.5	Measurement procedure	2572

A.11.5.1	Intra-frequency measurements	2572

A.11.5.1.1	Event-triggered reporting tests on PCC without gaps under non-DRX	2572

A.11.5.1.1.1	Test purpose and environment	2572

A.11.5.1.1.2	Test parameters	2572

A.11.5.1.1.3	Test Requirements	2573

A.11.5.1.2	Event-triggered reporting tests on PCC without gaps under DRX	2573

A.11.5.1.2.1	Test purpose and environment	2573

A.11.5.1.2.2	Test parameters	2573

A.11.5.1.2.3	Test Requirements	2573

A.11.5.1.3	Event-triggered reporting tests on PCC with per-UE gaps under non-DRX	2574

A.11.5.1.3.1	Test purpose and environment	2574

A.11.5.1.3.2	Test parameters	2574

A.11.5.1.3.3	Test Requirements	2574

A.11.5.1.4	Event-triggered reporting tests on PCC with per-UE gaps under DRX	2574

A.11.5.1.4.1	Test purpose and environment	2574

A.11.5.1.4.2	Test parameters	2574

A.11.5.1.4.3	Test Requirements	2574

A.11.5.1.5	Event-triggered reporting tests on SCC without gaps under non-DRX	2574

A.11.5.1.5.1	Test purpose and environment	2574

A.11.5.1.5.2	Test parameters	2575

A.11.5.1.6	Event-triggered reporting tests on SCC without gaps under DRX	2575

A.11.5.1.6.1	Test purpose and environment	2575

A.11.5.1.6.2	Test parameters	2575

A.11.5.1.6.3	Test Requirements	2575

A.11.5.1.7	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	2575

A.11.5.1.7.1	Test purpose and environment	2575

A.11.5.1.7.2	Test parameters	2575

A.11.5.1.7.3	Test Requirements	2575

A.11.5.1.8	Event-triggered reporting tests on SCC with per-UE gaps under DRX	2576

A.11.5.1.8.1	Test purpose and environment	2576

A.11.5.1.8.2	Test parameters	2576

A.11.5.1.8.3	Test Requirements	2576

A.11.5.1.9	RSSI measurement reporting on PCC	2576

A.11.5.1.9.1	Test purpose and environment	2576

A.11.5.1.9.2	Test parameters	2576

A.11.5.1.10	Channel occupancy measurement reporting on PCC	2577

A.11.5.1.10.1	Test purpose and environment	2577

A.11.5.1.10.2	Test parameters	2577

A.11.5.1.11	RSSI measurement reporting on SCC	2577

A.11.5.1.11.1	Test purpose and environment	2577

A.11.5.1.11.2	Test parameters	2577

A.11.5.1.12	Channel occupancy measurement reporting on SCC	2578

A.11.5.1.12.1	Test purpose and environment	2578

A.11.5.1.12.2	Test parameters	2578

A.11.5.2	Inter-frequency measurements	2578

A.11.5.2.1	RSSI measurement reporting	2578

A.11.5.2.1.1	Test purpose and environment	2578

A.11.5.2.1.2	Test parameters	2578

A.11.5.2.2	Channel occupancy measurement reporting	2579

A.11.5.2.2.1	Test purpose and environment	2579

A.11.5.2.2.2	Test parameters	2579

A.11.5.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	2579

A.11.5.2.3.1	Test Purpose and Environment	2579

A.11.5.2.3.2	Test Requirements	2583

A.11.5.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	2584

A.11.5.2.4.1	Test Purpose and Environment	2584

A.11.5.2.4.2	Test Requirements	2588

A.11.5.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	2589

A.11.5.2.5.1	Test Purpose and Environment	2589

A.11.5.2.5.2	Test Requirements	2593

A.11.5.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	2594

A.11.5.2.6.1	Test Purpose and Environment	2594

A.11.5.2.6.2	Test Requirements	2598

A.11.5.2.7	Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2599

A.11.5.2.7.1	Test Purpose and Environment	2599

A.11.5.2.7.2	Test Requirements	2603

A.11.5.2.8	Event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2604

A.11.5.2.8.1	Test Purpose and Environment	2604

A.11.5.2.8.2	Test Requirements	2608

A.11.5.2.9	Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2608

A.11.5.2.9.1	Test Purpose and Environment	2608

A.11.5.2.9.2	Test Requirements	2613

A.11.5.2.10	Event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2614

A.11.5.2.10.1	Test Purpose and Environment	2614

A.11.5.2.10.2	Test Requirements	2619

A.11.5.3	Inter-RAT E-UTRAN measurements	2620

A.11.5.4	L1-RSRP measurements for beam reporting	2620

A.11.5.4.1	SSB based L1-RSRP measurement when DRX is not used	2620

A.11.5.4.1.1	Test Purpose and Environment	2620

A.11.5.4.1.2	Test parameters	2620

A.11.5.4.1.3	Test Requirements	2622

A.11.5.4.2	SSB based L1-RSRP measurement when DRX is used	2622

A.11.5.4.2.1	Test Purpose and Environment	2622

A.11.5.4.2.2	Test parameters	2623

A.11.5.4.2.3	Test Requirements	2625

A.11.5.4.3	SSB based L1-RSRP measurement on SCC when DRX is not used	2625

A.11.5.4.3.1	Test Purpose and Environment	2625

A.11.5.4.3.2	Test parameters	2626

A.11.5.4.3.3	Test Requirements	2629

A.11.5.4.4	SSB based L1-RSRP measurement on SCC when DRX is used	2630

A.11.5.4.4.1	Test Purpose and Environment	2630

A.11.5.4.4.2	Test parameters	2630

A.11.5.4.4.3	Test Requirements	2633

A.11.6	Measurement performance	2634

A.11.6.1	SS-RSRP	2634

A.11.6.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	2634

A.11.6.1.1.1	Test Purpose and Environment	2634

A.11.6.1.1.2	Test parameters	2634

A.11.6.1.1.3	Test Requirements	2636

A.11.6.1.2	Intra-frequency measurement accuracy on SCC on a carrier frequency with CCA	2636

A.11.6.1.2.1	Test Purpose and Environment	2636

A.11.6.1.2.2	Test parameters	2636

A.11.6.1.2.3	Test Requirements	2638

A.11.6.2	SS-RSRQ	2638

A.11.6.3	SS-SINR	2638

A.11.6.4	L1-RSRP measurement for beam reporting with CCA serving cell	2638

A.11.6.4.1	SSB based L1-RSRP measurement	2638

A.11.6.4.1.1	Test Purpose and Environment	2638

A.11.6.4.1.2	Test parameters	2638

A.11.6.4.1.3	Test Requirements	2641

A.11.6.5	RSSI	2641

A.11.6.5.1 	Intra-frequency RSSI measurement accuracy on PCC with CCA	2641

A.11.6.5.1.1	Test Purpose and Environment	2641

A.11.6.5.1.2	Test parameters	2641

A.11.6.5.1.3	Test Requirements	2644

A.11.6.5.2 	Intra-frequency RSSI measurement accuracy on SCC with CCA	2644

A.11.6.5.2.1	Test Purpose and Environment	2644

A.11.6.5.2.2	Test parameters	2644

A.11.6.5.2.3	Test Requirements	2647

A.11.6.5.3 	Inter-frequency RSSI measurement accuracy on a carrier with CCA	2647

A.11.6.5.3.1	Test Purpose and Environment	2647

A.11.6.5.3.2	Test parameters	2647

A.11.6.5.3.3	Test Requirements	2650

A.11.6.6	Channel occupancy	2650

A.11.6.6.1 	Intra-frequency channel occupancy measurement accuracy on PCC with CCA	2650

A.11.6.6.1.1	Test Purpose and Environment	2650

A.11.6.6.1.2	Test parameters	2650

A.11.6.6.1.3	Test Requirements	2654

A.11.6.6.2 	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	2654

A.11.6.6.2.1	Test Purpose and Environment	2654

A.11.6.6.2.2	Test parameters	2654

A.11.6.6.2.3	Test Requirements	2657

A.11.6.6.3 	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	2657

A.11.6.6.3.1	Test Purpose and Environment	2657

A.11.6.6.3.2	Test parameters	2657

A.11.6.6.3.3	Test Requirements	2660

A.11.6.7	E-UTRAN RSRP	2660

A.11.6.8	E-UTRAN RSRQ	2660

A.11.6.9	E-UTRAN SINR	2660

A.12	E-UTRA Standalone Tests with at Least One NR Cell under CCA	2660

A.12.1	RRC_IDLE state mobility	2660

A.12.1.1	Inter-RAT cell re-selection to NR on a carrier frequency with CCA	2660

A.12.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1 when target cell is subject to CCA	2660

A.12.1.1.1.1	Test Purpose and Environment	2660

A.12.1.1.1.2	Test Requirements	2666

A.12.2	RRC_CONNECTED state mobility	2667

A.12.2.1	Handover	2667

A.12.2.1.1	E-UTRAN - NR with CCA handover	2667

A.12.2.1.1.1	Test Purpose and Environment	2667

A.12.2.1.1.2	Test Requirements	2672

A.12.3	Signalling characteristics	2673

A.12.3.1	Interruptions	2673

A.12.4	Measurement procedure	2673

A.12.4.1	E-UTRANNR inter-RAT SFTD measurements	2673

A.12.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay with NR under CCA in non-DRX	2673

A.12.4.1.1.1	Test Purpose and Environment	2673

A.12.4.1.1.2	Test Requirements	2677

A.12.4.2	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	2677

A.12.4.2.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2677

A.12.4.2.1.1	Test Purpose and Environment	2677

A.12.4.2.1.2	Test Requirements	2681

A.12.4.2.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2682

A.12.4.2.2.1	Test Purpose and Environment	2682

A.12.4.2.2.2	Test Requirements	2686

A.12.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2687

A.12.4.2.3.1	Test Purpose and Environment	2687

A.12.4.2.3.2	Test Requirements	2691

A.12.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2691

A.12.4.2.4.1	Test Purpose and Environment	2691

A.12.4.2.4.2	Test Requirements	2695

A.12.4.2.5	RSSI measurement reporting	2695

A.12.4.2.5.1	Test purpose and environment	2695

A.12.4.2.5.2	Test parameters	2695

A.12.4.2.6	Channel occupancy measurement reporting	2696

A.12.4.2.6.1	Test purpose and environment	2696

A.12.4.2.6.2	Test parameters	2696

A.12.5	Measurement performance	2696

A.12.5.1	E-UTRANNR SFTD	2696

A.12.5.2	E-UTRANNR SS-RSRP	2702

A.12.5.3	E-UTRANNR SS-RSRQ	2702

A.12.5.4	E-UTRANNR SS-SINR	2702

A.12.5.5	E-UTRANNR RSSI	2702

A.12.5.6	E-UTRANNR channel occupancy	2702

A.13	 NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	2702

A.13.1	Timing	2702

A.13.1.1	UE transmit timing	2702

A.13.1.2	Timing advance	2702

A.13.2	Signalling characteristics	2702

A.13.2.1	Interruption	2702

A.13.2.1.1	NR interruptions during SCell operations with CCA on SCell	2702

A.13.2.1.1.1	Test Purpose and Environment	2702

A.13.2.1.1.2	Test Requirements	2706

A.13.2.2	SCell activation and deactivation delay	2706

A.13.2.3	Active TCI state switching delay	2713

A.13.3	Measurement procedure	2713

A.13.3.1	Intra-frequency measurements	2713

A.13.3.1.1	Event-triggered reporting tests on SCC without gaps under non-DRX	2713

A.13.3.1.1.1	Test purpose and environment	2713

A.13.3.1.1.2	Test parameters	2713

A.13.3.1.1.3	Test Requirements	2718

A.13.3.1.2	Event-triggered reporting tests on SCC without gaps under DRX	2718

A.13.3.1.2.1	Test purpose and environment	2718

A.13.3.1.2.2	Test parameters	2718

A.13.3.1.2.3	Test Requirements	2722

A.13.3.1.3	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	2722

A.13.3.1.3.1	Test purpose and environment	2722

A.13.3.1.3.2	Test parameters	2722

A.13.3.1.3.3	Test Requirements	2727

A.13.3.1.4	Event-triggered reporting tests on SCC with per-UE gaps under DRX	2727

A.13.3.1.4.1	Test purpose and environment	2727

A.13.3.1.4.2	Test parameters	2727

A.13.3.1.4.3	Test Requirements	2732

A.13.3.1.5	RSSI measurement reporting on SCC	2732

A.13.3.1.5.1	Test purpose and environment	2732

A.13.3.1.5.2	Test parameters	2732

A.13.3.1.6	Channel occupancy measurement reporting on SCC	2734

A.13.3.1.6.1	Test purpose and environment	2734

A.13.3.1.6.2	Test parameters	2735

A.13.3.2	Inter-frequency measurements	2735

A.13.3.2.1	RSSI measurement reporting	2735

A.13.3.2.1.1	Test purpose and environment	2735

A.13.3.2.1.2	Test parameters	2735

A.13.3.2.2	Channel occupancy measurement reporting	2736

A.13.3.2.2.1	Test purpose and environment	2736

A.13.3.2.2.2	Test parameters	2736

A.13.3.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	2736

A.13.3.2.3.1	Test Purpose and Environment	2736

A.13.3.2.3.2	Test Requirements	2742

A.13.3.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	2742

A.13.3.2.4.1	Test Purpose and Environment	2742

A.13.3.2.4.2	Test Requirements	2747

A.13.3.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	2748

A.13.3.2.5.1	Test Purpose and Environment	2748

A.13.3.2.5.2	Test Requirements	2753

A.13.3.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	2753

A.13.3.2.6.1	Test Purpose and Environment	2753

A.13.3.2.6.2	Test Requirements	2759

A.13.3.3	L1-RSRP measurements for beam reporting	2760

A.13.3.3.1	SSB based L1-RSRP measurement when DRX is not used	2760

A.13.3.3.1.1	Test Purpose and Environment	2760

A.13.3.3.1.2	Test parameters	2760

A.13.3.3.1.3	Test Requirements	2764

A.13.3.3.2	SSB based L1-RSRP measurement when DRX is used	2764

A.13.3.3.2.1	Test Purpose and Environment	2764

A.13.3.3.2.2	Test parameters	2764

A.13.3.3.2.3	Test Requirements	2768

A.13.4	Measurement performance	2768

A.13.4.1	SS-RSRP	2768

A.13.4.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	2768

A.13.4.1.1.1	Test Purpose and Environment	2768

A.13.4.1.1.2	Test parameters	2768

A.13.4.1.1.3	Test Requirements	2770

A.13.4.2	SS-RSRQ	2770

A.13.4.3	SS-SINR	2770

A.13.4.4	L1-RSRP measurement for beam reporting with CCA serving cell	2770

A.13.4.4.1	SSB based L1-RSRP measurement	2770

A.13.4.4.1.1	Test Purpose and Environment	2770

A.13.4.4.1.2	Test parameters	2770

A.13.4.4.1.3	Test Requirements	2775

A.13.4.5	RSSI	2775

A.13.4.5.1 	Intra-frequency RSSI measurement accuracy on a carrier with CCA	2775

A.13.4.5.1.1	Test Purpose and Environment	2775

A.13.4.5.1.2	Test parameters	2775

A.13.4.5.1.3	Test Requirements	2779

A.13.4.5.2	  Inter-frequency RSSI measurement accuracy on a carrier with CCA	2779

A.13.4.5.2.1	Test Purpose and Environment	2779

A.13.4.5.2.2	Test parameters	2779

A.13.4.5.2.3	Test Requirements	2783

A.13.4.6	Channel occupancy	2783

A.13.4.6.1 	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	2783

A.13.4.6.1.1	Test Purpose and Environment	2783

A.13.4.6.1.2	Test parameters	2783

A.13.4.6.1.3	Test Requirements	2787

A.13.4.6.2	  Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	2787

A.13.4.6.2.1	Test Purpose and Environment	2787

A.13.4.6.2.2	Test parameters	2787

A.13.4.6.2.3	Test Requirements	2791

Annex B (normative): Conditions for RRM requirements applicability for operating bands	1818

B.1	Conditions for NR RRC_IDLE state mobility	1818

B.1.1	Introduction	1818

B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection	1818

B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection	1819

B.2	Conditions for UE measurements procedures and performance requirements in RRC_CONNECTED state	1820

B.2.1	Introduction	1820

B.2.1.1	General	1820

B.2.1.2	Derivation of Minimum SSB_RP values for FR1	1820

B.2.1.3	Derivation of Minimum SSB_RP values for FR2	1820

B.2.1.3.1	Minimum SSB_RP values for Rx Beam Peak angle of arrival	1820

B.2.1.3.2	Minimum SSB_RP values for angle of arrival within Spherical coverage	1821

B.2.1.4	Gain to SS-RSRP and CSI-RSRP measurement point for FR1	1821

B.2.1.5	Gain to SS-RSRP and CSI-RSRP measurement point for FR2	1822

B.2.1.5.1	Gain to SS-RSRP and CSI-RSRP measurement point for Rx Beam Peak angle of arrival	1822

B.2.1.6	Gain to PRS-RSRP measurement point for FR2	1822

B.2.1.6.1	Gain to PRS-RSRP measurement point for Rx Beam Peak angle of arrival	1822

B.2.2	Conditions for NR intra-frequency measurements	1823

B.2.3	Conditions for NR inter-frequency measurements	1824

B.2.4	Conditions for NR L1-RSRP reporting	1826

B.2.4.1	Conditions for SSB based L1-RSRP reporting	1826

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting	1827

B.2.5	Conditions for RRC connection release with redirection to NR	1828

B.2.6	Void	1829

B.2.6.1	Void	1829

B.2.6.2	Void	1830

B.2.7	Conditions for SRS-RSRP measurements	1830

B.2.8	Conditions for NR L1-SINR reporting	1830

B.2.8.1	Conditions for L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	1830

B.2.8.2	Conditions for L1-SINR reporting with SSB based CMR and dedicated IMR configured	1831

B.2.8.2.1	L1-SINR reporting with SSB based CMR and dedicated ZP-IMR configured	1831

B.2.8.2.2	L1-SINR reporting with SSB based CMR and dedicated NZP-IMR configured	1832

B.2.8.3	Conditions for L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	1834

B.2.8.3.1	L1-SINR reporting with CSI-RS based CMR and dedicated ZP-IMR configured	1834

B.2.8.3.2	L1-SINR reporting with CSI-RS based CMR and dedicated NZP-IMR configured	1835

B.2.9	Conditions for NR intra-frequency measurements under CCA	1836

B.2.10	Conditions for NR inter-frequency measurements under CCA	1836

B.2.11	Conditions for NR L1-RSRP reporting under CCA	1836

B.2.11.1	Conditions for SSB based L1-RSRP reporting	1836

B.2.12	Conditions for NR CSI-RS based intra-frequency measurements	1837

B.2.13	Conditions for NR CSI-RS based inter-frequency measurements	1838

B.2.14	Conditions for NR PRS-based measurements	1840

B.3	RRM Requirements Exceptions	1841

B.3.1	Introduction	1841

B.3.2	Receiver sensitivity relaxation for CA	1841

B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1	1841

B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1	1842

B.3.2.2.1	Inter-band carrier aggregation	1842

B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA	1842

B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	1842

B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2	1842

B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2	1842

B.3.2.4.1	Intra-band contiguous carrier aggregation	1842

B.3.2.4.2	Intra-band non-contiguous carrier aggregation	1842

B.3.3	Receiver sensitivity relaxation for DC	1843

B.3.3.1	Receiver sensitivity relaxation for EN-DC	1843

B.3.3.2	Receiver sensitivity relaxation for NE-DC	1843

B.3.4	Receiver sensitivity relaxation for SUL	1843

B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1	1843

B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1	1843

B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL	1843

B.4	Conditions for V2X	1843

B.4.1	Test parameters for GNSS signals	1843

B.4.2	Conditions for PSBCH-RSRP Accuracy Requirements	1844

B.4.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	1844

B.4.4	Conditions for L1 SL-RSRP Accuracy Requirements	1844

Annex C (informative): Change history	1846