| 3GPP TS 38.133 V16.12.0 (2022-06) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Requirements for support of radio resource management (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	42

1	Scope	44

2	References	44

3	Definitions, symbols and abbreviations	45

3.1	Definitions	45

3.2	Symbols	46

3.3	Abbreviations	47

3.4	Test tolerances	50

3.5	Frequency bands grouping	50

3.5.1	Introduction	50

3.5.2	NR operating bands in FR1	50

3.5.3	NR operating bands in FR2	51

3.6	Applicability of requirements in this specification version	51

3.6.1	RRC connected state requirements in DRX	52

3.6.2	Number of serving carriers	52

3.6.2.1	Number of serving carriers for SA	52

3.6.2.2	Number of serving carriers for EN-DC	52

3.6.2.3	Number of serving carriers for NE-DC	52

3.6.2.4	Number of serving carriers for NR-DC	53

3.6.3	Applicability for intra-band FR2	53

3.6.4	Applicability for FR2 UE power classes	53

3.6.5	Applicability for SDL bands	53

3.6.6	Applicability of requirements for NGEN-DC operation	53

3.6.7	Applicability of QCL	53

3.6.9	Applicability of requirements for scheduling availability	54

3.6.10	Applicability of requirements for measurement restrictions	54

4	SA: RRC_IDLE state mobility	54

4.1	Cell Selection	54

4.2	Cell Re-selection	54

4.2.1	Introduction	54

4.2.2	Requirements	55

4.2.2.1	UE measurement capability	55

4.2.2.2	Measurement and evaluation of serving cell	55

4.2.2.3	Measurements of intra-frequency NR cells	56

4.2.2.4	Measurements of inter-frequency NR cells	57

4.2.2.5	Measurements of inter-RAT E-UTRAN cells	59

4.2.2.6	Maximum interruption in paging reception	61

4.2.2.7	General requirements	61

4.2.2.8	Minimum requirement at transitions	61

4.2.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	62

4.2.2.9.1	Introduction	62

4.2.2.9.2	Measurements for UE fulfilling low mobility criterion	62

4.2.2.9.3	Measurements for UE fulfilling not-at-cell edge criterion	63

4.2.2.9.4	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	64

4.2.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	64

4.2.2.10.1	Introduction	64

4.2.2.10.2	Measurements for UE fulfilling low mobility criterion	64

4.2.2.10.3	Measurements for UE fulfilling not-at-cell edge criterion	65

4.2.2.10.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	66

4.2.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	67

4.2.2.11.1	Introduction	67

4.2.2.11.2	Measurements for UE fulfilling low mobility criterion	67

4.2.2.11.3	Measurements for UE fulfilling with not-at-cell edge criterion	68

4.2.2.11.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	69

4.2A	Cell Re-selection when subject to CCA	69

4.2A.1	Introduction	69

4.2A.2	Requirements	70

4.2A.2.1	UE measurement capability	70

4.2A.2.2	Measurement and evaluation when subject to CCA on the serving cell	70

4.2A.2.3	Measurements of intra-frequency NR cells when subject to CCA on the serving cell and target cell	71

4.2A.2.4	Measurements of inter-frequency NR cells when subject to CCA on the target cell	72

4.2A.2.5	Measurements of inter-RAT E-UTRAN cells when subject to CCA on the serving cell	74

4.2A.2.6	Maximum interruption in paging reception when subject to CCA on the target cell	74

4.2A.2.7	General requirements	74

4.3	Minimization of Drive Tests (MDT)	75

4.3.1	Introduction	75

4.3.2	Measurement Requirements	75

4.3.3	Requirements for Relative Time Stamp Accuracy	75

4.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	76

4.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	76

4.4	Idle Mode CA/DC Measurements	76

4.4.1	Introduction	76

4.4.2	Measurement Requirements	76

4.4.2.1	Detected cell requirement during state transition and Idle mode	76

4.4.2.2	Measurements of inter-frequency CA/DC candidate cells	77

4.4.2.3	Measurements on serving cell	78

4.4.2.4	Measurements of E-UTRAN inter-RAT DC candidate cells	78

5	SA: RRC_INACTIVE state mobility	80

5.1	Cell Re-selection	80

5.1.1	Introduction	80

5.1.2	Requirements	80

5.1.2.1	UE measurement capability	80

5.1.2.2	Measurement and evaluation of serving cell	80

5.1.2.3	Measurements of intra-frequency NR cells	80

5.1.2.4	Measurements of inter-frequency NR cells	80

5.1.2.5	Measurements of inter-RAT E-UTRAN cells	80

5.1.2.6	Maximum interruption in paging reception	80

5.1.2.7	General requirements	80

5.1A	Cell Re-selection with CCA	80

5.1A.1	Introduction	80

5.1A.2	Requirements	81

5.1A.2.1	UE measurement capability	81

5.1A.2.2	Measurement and evaluation when CCA is used on the serving cell	81

5.1A.2.3	Measurements of intra-frequency NR cells when CCA is used on the serving cell and target cell	81

5.1A.2.4	Measurements of inter-frequency NR cells when CCA is used on the target cell	81

5.1A.2.5	Measurements of inter-RAT E-UTRAN cells when CCA is used on the serving cell	81

5.1A.2.6	Maximum interruption in paging reception when CCA is used on the target cell	81

5.1A.2.7	General requirements	81

5.2	Void	81

5.3	Minimization of Drive Tests (MDT)	81

5.3.1	Introduction	81

5.3.2	Measurement Requirements	82

5.3.3	Requirements for Relative Time Stamp Accuracy	82

5.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	82

5.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	82

5.3.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	82

5.4	Idle Mode CA/DC Measurements	82

5.4.1	Introduction	82

5.4.2	Measurement Requirements	82

5.4.2.1	Detected cell requirement during state transition and Idle mode	83

5.4.2.2	Measurements of inter-frequency CA/DC candidate cells	83

5.4.2.3	Measurements on serving cell	83

5.4.2.4	Measurements on E-UTRAN inter-RAT DC candidate cells	83

6	RRC_CONNECTED state mobility	83

6.1	Handover	83

6.1.1	NR Handover	83

6.1.1.1	Introduction	83

6.1.1.2	NR FR1 - NR FR1 Handover	83

6.1.1.2.1	Handover delay	83

6.1.1.2.2	Interruption time	83

6.1.1.3	NR FR2- NR FR1 Handover	84

6.1.1.3.1	Handover delay	84

6.1.1.3.2	Interruption time	84

6.1.1.4	NR FR2- NR FR2 Handover	85

6.1.1.4.1	Handover delay	85

6.1.1.4.2	Interruption time	85

6.1.1.5	NR FR1- NR FR2 Handover	86

6.1.1.5.1	Handover delay	86

6.1.1.5.2	Interruption time	86

6.1.2	NR Handover to other RATs	87

6.1.2.1	NR – E-UTRAN Handover	87

6.1.2.1.1	Introduction	87

6.1.2.1.2	Handover delay	87

6.1.2.1.3	Interruption time	87

6.1.2.2	NR – UTRAN Handover	88

6.1.2.2.1	Introduction	88

6.1.2.2.2	Handover delay	88

6.1.2.2.3	Interruption time	88

6.1.3	NR DAPS Handover	89

6.1.3.1	Introduction	89

6.1.3.2	NR FR1 - NR FR1 DAPS Handover	89

6.1.3.2.1	DAPS handover delay	89

6.1.3.2.2	Interruption time	90

6.1.3.3	NR FR2- NR FR1 DAPS Handover	92

6.1.3.3.1	DAPS handover delay	92

6.1.3.3.2	Interruption time	92

6.1.3.4	NR FR1- NR FR2 DAPS Handover	93

6.1.3.4.1	DAPS handover delay	93

6.1.3.4.2	Interruption time	93

6.1.4	NR Conditional Handover	94

6.1.4.1	Introduction	94

6.1.4.2	NR FR1 – NR FR1 conditional handover	94

6.1.4.3	NR FR2 – NR FR1 conditional handover	95

6.1.4.4	NR FR2 – NR FR2 conditional handover	96

6.1.4.4.1	Handover delay	96

6.1.4.4.2	Measurement time	96

6.1.4.4.3	Preparation time	96

6.1.4.4.4	Interruption time	96

6.1.4.5	NR FR1 – NR FR2 conditional handover	97

6.1A	Void	97

6.1A.1	Void	97

6.1A.1.1	Void	97

6.1A.1.2	Void	97

6.1A.1.2.1	Void	97

6.1A.1.2.2	Void	97

6.1B	Handover to target cell using CCA	97

6.1B.1	NR Handover	97

6.1B.1.1	Introduction	97

6.1B.1.2	NR FR1 - NR FR1 Handover	98

6.1B.1.2.1	Handover delay	98

6.1B.1.2.2	Interruption time	98

6.2	RRC Connection Mobility Control	99

6.2.1	SA: RRC Re-establishment	99

6.2.1.1	Introduction	99

6.2.1.2	Requirements	99

6.2.1.2.1	UE Re-establishment delay requirement	99

6.2.1A	RRC Re-establishment with CCA	100

6.2.1A.1	Introduction	100

6.2.1A.2	Requirements	101

6.2.1A.2.1	UE Re-establishment with CCA delay requirement	101

6.2.2	Random access	103

6.2.2.1	Introduction	103

6.2.2.2	Requirements for 4-step RA type	103

6.2.2.2.1	Contention based random access	103

6.2.2.2.2	Non-Contention based random access	104

6.2.2.2.3	UE behaviour when configured with supplementary UL	105

6.2.2.3	Requirements for 2-step RA type	105

6.2.2.3.1	Contention based random access	105

6.2.2.3.2	Non-Contention based random access	106

6.2.2.3.3	UE behaviour when configured with supplementary UL	107

6.2.2A	Random access when CCA is used on target frequency	107

6.2.2A.1	Introduction	107

6.2.2A.2	Requirements for 4-step RA type	107

6.2.2A.2.1	Contention based random access	107

6.2.2A.2.2	Non-Contention based random access	108

6.2.2A.3	Requirements for 2-step RA type	109

6.2.2A.3.1	Contention based random access	110

6.2.2A.3.2	Non-Contention based random access	111

6.2.3	SA: RRC Connection Release with Redirection	112

6.2.3.1	Introduction	112

6.2.3.2	Requirements	112

6.2.3.2.1	RRC connection release with redirection to NR	112

6.2.3.2.2	RRC connection release with redirection to E-UTRAN	113

6.2.3.2.3	RRC connection release with redirection to NR carrier subject to CCA	113

7	Timing	114

7.1	UE transmit timing	114

7.1.1	Introduction	114

7.1.2	Requirements	115

7.1.2.1	Gradual timing adjustment	116

7.1.2.2	Void	116

7.2	UE timer accuracy	116

7.2.1	Introduction	116

7.2.2	Requirements	117

7.3	Timing advance	117

7.3.1	Introduction	117

7.3.2	Requirements	117

7.3.2.1	Timing Advance adjustment delay	117

7.3.2.2	Timing Advance adjustment accuracy	117

7.4	Cell phase synchronization accuracy	117

7.4.1	Definition	117

7.4.2	Minimum requirements	117

7.5	Maximum Transmission Timing Difference	118

7.5.1	Introduction	118

7.5.2	Minimum Requirements for inter-band EN-DC	118

7.5.2.1	Minimum Requirements for inter-band synchronous EN-DC	118

7.5.3	Minimum Requirements for intra-band EN-DC	119

7.5.4	Minimum Requirements for NR Carrier Aggregation	119

7.5.5	Minimum Requirements for inter-band NE-DC	119

7.5.5.1	Minimum Requirements for inter-band synchronous NE-DC	120

7.5.6	Minimum Requirements for inter-band NR DC	120

7.6	Maximum Receive Timing Difference	121

7.6.1	Introduction	121

7.6.2	Minimum Requirements for inter-band EN-DC	121

7.6.2.1	Minimum Requirements for inter-band synchronous EN-DC	121

7.6.3	Minimum Requirements for intra-band EN-DC	122

7.6.4	Minimum Requirements for NR Carrier Aggregation	122

7.6.5	Minimum Requirements for inter-band NE-DC	123

7.6.5.1	Minimum Requirements for inter-band synchronous NE-DC	123

7.6.6	Minimum Requirements for inter-band NR DC	124

7.7	deriveSSB-IndexFromCell tolerance	124

7.7.1	Minimum requirements	124

7.8	Void	125

8	Signalling characteristics	125

8.1	Radio Link Monitoring	125

8.1.1	Introduction	125

8.1.2	Requirements for SSB based radio link monitoring	126

8.1.2.1	Introduction	126

8.1.2.2	Minimum requirement	127

8.1.2.3	Measurement restrictions for SSB based RLM	128

8.1.3	Requirements for CSI-RS based radio link monitoring	129

8.1.3.1	Introduction	129

8.1.3.2	Minimum requirement	130

8.1.3.3	Measurement restrictions for CSI-RS based RLM	132

8.1.4	Minimum requirement at transitions	132

8.1.5	Minimum requirement for UE turning off the transmitter	133

8.1.6	Minimum requirement for L1 indication	133

8.1.7	Scheduling availability of UE during radio link monitoring	133

8.1.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	133

8.1.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	133

8.1.7.3	Scheduling availability of UE performing radio link monitoring on FR2	134

8.1.7.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	134

8.1A	Radio Link Monitoring with CCA on Target Frequency	134

8.1A.1	Introduction	134

8.1A.2	Requirements for SSB Based Radio Link Monitoring	135

8.1A.2.1	Introduction	135

8.1A.2.2	Minimum Requirement	136

8.1A.3	Minimum requirement at transitions	137

8.1A.4	Minimum requirement for UE turning off the transmitter	138

8.1A.5	Minimum requirement for L1 indication	138

8.1A.6	Scheduling availability of UE during radio link monitoring	138

8.1A.6.1	Scheduling availability of UE performing radio link monitoring with the same subcarrier spacing as PDSCH/PDCCH	138

8.1A.6.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH	138

8.2	Interruption	139

8.2.1	EN-DC Interruption	139

8.2.1.1	Introduction	139

8.2.1.2	Requirements	139

8.2.1.2.1	Interruptions at transitions between active and non-active during DRX	139

8.2.1.2.2	Interruptions at transitions from non-DRX to DRX	140

8.2.1.2.3	Interruptions at SCell addition/release	140

8.2.1.2.4	Interruptions at SCell activation/deactivation	141

8.2.1.2.5	Interruptions during measurements on SCC	142

8.2.1.2.6	Interruptions at UL carrier RRC reconfiguration	143

8.2.1.2.7	Interruptions due to Active BWP switching Requirement	144

8.2.1.2.8	Interruptions at direct SCell activation and hibernation	145

8.2.1.2.9	Interruptions at SCell hibernation	145

8.2.1.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	146

8.2.1.2.11	Interruptions due to UE-specific CBW change	146

8.2.1.2.12	 Interruptions at NR SRS carrier based switching	146

8.2.1.2.13	 Interruptions at E-UTRA SRS carrier based switching	147

8.2.1.2.14	DL Interruptions at switching between two uplink carriers	148

8.2.1.2.15	Interruptions due to SCell dormancy	149

8.2.1.2.16	Interruptions when identifying CGI of an NR cell with autonomous gaps	149

8.2.1.2.17	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	150

8.2.2	SA: Interruptions with Standalone NR Carrier Aggregation	150

8.2.2.1	Introduction	150

8.2.2.2	Requirements	151

8.2.2.2.1	Interruptions at SCell addition/release	151

8.2.2.2.2	Interruptions at SCell activation/deactivation	152

8.2.2.2.3	Interruptions during measurements on deactivated SCC	153

8.2.2.2.4	Interruptions at UL carrier RRC reconfiguration	154

8.2.2.2.5	Interruptions due to Active BWP switching Requirement	154

8.2.2.2.6	Interruptions at inter-frequency SFTD measurement	155

8.2.2.2.7	Interruptions at SCell activation/deactivation with multiple downlink SCells	156

8.2.2.2.8	Interruptions due to UE-specific CBW change	156

8.2.2.2.9	 Interruptions at NR SRS carrier based switching	157

8.2.2.2.10	DL Interruptions at UE switching between two uplink carriers	158

8.2.2.2.11	Interruptions at direct SCell activation	159

8.2.2.2.12	Interruptions due to SCell dormancy	159

8.2.2.2.13	Interruptions at transitions between active and non-active during DRX	159

8.2.2.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	160

8.2.2.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	160

8.2.3	NE-DC Interruptions	161

8.2.3.1	Introduction	161

8.2.3.2	Requirements	161

8.2.3.2.1	Interruptions at transitions between active and non-active during DRX	161

8.2.3.2.2	Interruptions at transitions from non-DRX to DRX	162

8.2.3.2.3	Interruptions at PSCell/SCell addition/release	162

8.2.3.2.4	Interruptions at SCell activation/deactivation	163

8.2.3.2.5	Interruptions during measurements on SCC	164

8.2.3.2.6	Interruptions at UL carrier RRC reconfiguration	165

8.2.3.2.7	Interruptions due to Active BWP switching Requirement	166

8.2.3.2.8	Interruptions at direct SCell activation and hibernation	166

8.2.3.2.9	Interruptions at SCell hibernation	166

8.2.3.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	167

8.2.3.2.11	 Interruptions at NR SRS carrier based switching	167

8.2.3.2.12	 Interruptions at E-UTRA SRS carrier based switching	168

8.2.3.2.13	Interruptions due to SCell dormancy	169

8.2.3.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	170

8.2.3.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	170

8.2.3.2.16	Interruptions due to UE-specific CBW change	171

8.2.4	NR-DC: Interruptions	171

8.2.4.1	Introduction	171

8.2.4.2	Requirements	172

8.2.4.2.1	Interruptions at PSCell/SCell addition/release	172

8.2.4.2.2	Interruptions at SCell activation/deactivation	173

8.2.4.2.3	Interruptions during measurements on SCC	174

8.2.4.2.4	Interruptions at UL carrier RRC reconfiguration	174

8.2.4.2.5	Interruptions due to Active BWP switching Requirement	174

8.2.4.2.6	Interruptions at transitions between active and non-active during DRX	175

8.2.4.2.7	Interruptions at transitions from non-DRX to DRX	175

8.2.4.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	175

8.2.4.2.9	Interruptions at NR SRS carrier based switching	175

8.2.4.2.10	Interruptions at direct SCell activation	177

8.2.4.2.11	Interruptions when identifying CGI of an NR cell with autonomous gaps	177

8.2.4.2.12	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	178

8.2.4.2.13	 Interruptions due to SCell dormancy	178

8.2.4.2.14	Interruptions due to UE-specific CBW change	179

8.2.4.2A	Void	179

8.2.4.2A.1	Void	179

8.2.4.2A.2	Void	179

8.2.4.2A.3	Void	179

8.3	SCell Activation and Deactivation Delay	179

8.3.1	Introduction	179

8.3.2	SCell Activation Delay Requirement for Deactivated SCell	179

8.3.3	SCell Deactivation Delay Requirement for Activated SCell	183

8.3.4	Direct SCell Activation at SCell addition	184

8.3.5	Direct SCell Activation at Handover	185

8.3.7	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	187

8.3.8	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	191

8.3.9	Direct SCell Activation of Multiple Downlink SCells at SCell addition	191

8.3.10	Direct SCell Activation of Multiple Downlink SCells at Handover	193

8.3A	SCell Activation and Deactivation Delay in Carriers with CCA	194

8.3A.1	Introduction	194

8.3A.2	SCell Activation Delay Requirement for Deactivated SCell	194

8.3A.3	SCell Deactivation Delay Requirement for Activated SCell	197

8.4	UE UL carrier RRC reconfiguration delay	197

8.4.1	Introduction	197

8.4.2	UE UL carrier configuration delay requirement	198

8.4.3	UE UL carrier deconfiguration delay requirement	198

8.5	Link Recovery Procedures	198

8.5.1	Introduction	198

8.5.2	Requirements for SSB based beam failure detection	199

8.5.2.1	Introduction	199

8.5.2.2	Minimum requirement	199

8.5.2.3	Measurement restriction for SSB based beam failure detection	201

8.5.3	Requirements for CSI-RS based beam failure detection	201

8.5.3.1	Introduction	201

8.5.3.2	Minimum requirement	202

8.5.3.3	Measurement restrictions for CSI-RS beam failure detection	204

8.5.4	Minimum requirement for L1 indication	205

8.5.5	Requirements for SSB based candidate beam detection	205

8.5.5.1	Introduction	205

8.5.5.2	Minimum requirement	205

8.5.5.3	Measurement restriction for SSB based candidate beam detection	207

8.5.6	Requirements for CSI-RS based candidate beam detection	208

8.5.6.1	Introduction	208

8.5.6.2	Minimum requirement	208

8.5.6.3	Measurement restriction for CSI-RS based candidate beam detection	210

8.5.7	Scheduling availability of UE during beam failure detection	211

8.5.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	211

8.5.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	211

8.5.7.3	Scheduling availability of UE performing beam failure detection on FR2	211

8.5.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR DC	212

8.5.8	Scheduling availability of UE during candidate beam detection	212

8.5.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	212

8.5.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	212

8.5.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	212

8.5.8.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	213

8.5.9	Requirements for Beam Failure Recovery in SCell	213

8.5.9.1	Introduction	213

8.5.9.2	Requirement	213

8.5.10	Minimum requirement at transitions for beam failure detection	214

8.5A	Link Recovery Procedures when CCA is used on target frequency	214

8.5A.1	Introduction	214

8.5A.2	Requirements for SSB based beam failure detection	215

8.5A.2.1	Introduction	215

8.5A.2.2	Minimum requirement	215

8.5A.2.3	Measurement restriction for SSB based beam failure detection	216

8.5A.4	Minimum requirement for L1 indication	216

8.5A.5	Requirements for SSB based candidate beam detection	216

8.5A.5.1	Introduction	216

8.5A.5.2	Minimum requirement	216

8.5A.5.3	Measurement restriction for SSB based candidate beam detection	217

8.5A.7	Scheduling availability of UE during beam failure detection	217

8.5A.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH	217

8.5A.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH	217

8.5A.8	Scheduling availability of UE during candidate beam detection	217

8.5A.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH	218

8.5A.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH	218

8.6	Active BWP switch delay	218

8.6.1	Introduction	218

8.6.2	DCI and timer based BWP switch delay on a single CC	218

8.6.2A	DCI based BWP switch delay on multiple CCs	219

8.6.2A.1	Simultaneous DCI based BWP switch delay on multiple CCs	219

8.6.2A.2	Non-simultaneous DCI based BWP switch delay on multiple CCs	221

8.6.2B	Timer based BWP switch delay on multiple CCs	221

8.6.2B.1	Simultaneous timer based BWP switch delay on multiple CCs	221

8.6.2B.2	Non-simultaneous timer based BWP switch delay on multiple CCs	222

8.6.3	RRC based BWP switch delay on a single CC	222

8.6.3A	RRC based BWP switch delay on multiple CCs	223

8.6.3A.1	Simultaneous RRC based BWP switch delay on multiple CCs	223

8.6.3A.2	Non-simultaneous RRC based BWP switch delay on multiple CCs	223

8.6.4	BWP switch delay on Consistent UL CCA recovery	224

8.7	Void	224

8.8	NE-DC: E-UTRAN PSCell Addition and Release Delay	224

8.8.1	Introduction	224

8.8.2	E-UTRAN PSCell Addition Delay Requirement	224

8.8.3	E-UTRAN PSCell Release Delay Requirement	225

8.9	NR-DC: PSCell Addition and Release Delay	225

8.9.1	Introduction	225

8.9.2	PSCell Addition Delay Requirement	225

8.9.3	PSCell Release Delay Requirement	226

8.10	Active TCI state switching delay	226

8.10.4	DCI based TCI state switch delay	228

8.10.5	RRC based TCI state switch delay	228

8.10.6	Active TCI state list update delay	228

8.10A	Active TCI state switching delay with CCA	229

8.10A.1	Introduction	229

8.10A.2	Known conditions for TCI state	229

8.10A.3	MAC-CE based TCI state switch delay	229

8.10A.4	DCI based TCI state switch delay	230

8.10A.5	RRC based TCI state switch delay	230

8.10A.6	Active TCI state list update delay	231

8.11	PSCell Change	231

8.11A	void	232

8.11B	Conditional PSCell Change	232

8.11B.1	Introduction	232

8.11B.2	Conditoinal PSCell Change delay	232

8.11B.2.1	Measurement time	232

8.12	Uplink spatial relation switch delay	233

8.12.1	Introduction	233

8.12.2	Known conditions for spatial relation when associated with DL-RS	233

8.12.3	MAC-CE based spatial relation switch delay	233

8.12.4	DCI based spatial relation switch delay	234

8.12.5	RRC based spatial relation switch delay	234

8.13	UE-specific CBW change	235

8.13.1	Introduction	235

8.13.2	UE-specific CBW change delay	235

8.14	Pathloss reference signal switching delay	235

8.14.1	Introduction	235

8.14.2	Known conditions for pathloss reference signal	235

8.14.3	MAC-CE based pathloss reference signal switch delay	236

9	Measurement Procedure	236

9.1	General measurement requirement	236

9.1.1	Introduction	236

9.1.2	Measurement gap	237

9.1.2.1	EN-DC: Measurement Gap Sharing	248

9.1.2.1a	SA: Measurement Gap Sharing	248

9.1.2.1b	NE-DC: Measurement Gap Sharing	249

9.1.2.1c	NR-DC: Measurement Gap Sharing	250

9.1.3	UE Measurement capability	251

9.1.3.1	EN-DC: Monitoring of multiple layers using gaps	251

9.1.3.1a	SA: Monitoring of multiple layers using gaps	251

9.1.3.1b	NE-DC: Monitoring of multiple layers using gaps	252

9.1.3.1c	NR-DC: Monitoring of multiple layers using gaps	252

9.1.3.2	EN-DC: Maximum allowed layers for multiple monitoring	253

9.1.3.2a	SA: Maximum allowed layers for multiple monitoring	254

9.1.3.2b	NE-DC: Maximum allowed layers for multiple monitoring	254

9.1.3.2c	NR-DC: Maximum allowed layers for multiple monitoring	255

9.1A.3.2	Void	256

9.1.3A	UE Measurement capability under operation mode with CCA	256

9.1.3A.1	EN-DC: Monitoring of multiple layers using gaps under CCA	256

9.1.3A.1A	SA: Monitoring of multiple layers using gaps under CCA	256

9.1.3A.2	EN-DC: Maximum allowed layers for multiple monitoring under CCA	256

9.1A.3.2a	Void	257

9.1.3A.2A	SA: Maximum allowed layers for multiple monitoring under CCA	257

9.1.4	Capabilities for Support of Event Triggering and Reporting Criteria	257

9.1.4.1	Introduction	257

9.1.4.2	Requirements	257

9.1.5	Carrier-specific scaling factor	261

9.1.5.1	Monitoring of multiple layers outside gaps	261

9.1.5.1.1	EN-DC mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	263

9.1.5.1.2	SA mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	264

9.1.5.1.3	NR-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed outside gaps	265

9.1.5.1.4	NE-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based measurements performed outside gaps	266

9.1.5.2	Monitoring of multiple layers within gaps	267

9.1.5.2.1	EN-DC mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	268

9.1.5.2.2	SA mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	270

9.1.5.2.3	NE-DC: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed within gaps	271

9.1.5.2.4	NR-DC: carrier-specific scaling factor for SSB-based and CSI-RS-based L3 measurements performed within gaps	273

9.1.5.2.5	SA mode: carrier-specific scaling factor for PRS-based measurements performed within gaps	275

9.1.5.2.6	NE-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	275

9.1.5.2.7	NR-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	275

9.1.6	Minimum requirement at transitions	275

9.2	NR intra-frequency measurements	276

9.2.1	Introduction	276

9.2.2	Requirements applicability	276

9.2.3	Number of cells and number of SSB	277

9.2.3.1	Requirements for FR1	277

9.2.3.2	Requirements for FR2	277

9.2.4	Measurement Reporting Requirements	277

9.2.4.1	Periodic Reporting	277

9.2.4.2	Event-triggered Periodic Reporting	277

9.2.4.3	Event Triggered Reporting	277

9.2.5	Intrafrequency measurements without measurement gaps	278

9.2.5.1	Intrafrequency cell identification	278

9.2.5.2	Measurement period	281

9.2.5.3	Scheduling availability of UE during intra-frequency measurements	282

9.2.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	283

9.2.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	283

9.2.5.3.3	Scheduling availability of UE performing measurements on FR2	283

9.2.5.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	284

9.2.5.4	SFTD Measurements between PCell and PSCell	284

9.2.5.4.1	Introduction	284

9.2.5.4.2	SFTD Measurement delay	285

9.2.5.4.3	SFTD Measurement Reporting Delay	285

9.2.6	Intra-frequency measurements with measurement gaps	285

9.2.6.1	Void	285

9.2.6.2	Intra-frequency cell identification	285

9.2.6.3	Intrafrequency Measurement Period	287

9.2A	NR intra-frequency measurements with CCA	288

9.2A.1	Introduction	288

9.2A.2	Requirements applicability	289

9.2A.3	Number of cells and number of SSB	289

9.2A.4	Measurement Reporting Requirements	289

9.2A.5	Intra-frequency measurements without measurement gaps	290

9.2A.5.2	Measurement period	292

9.2A.5.3	Scheduling availability of UE during intra-frequency measurements	293

9.2A.5.3.1	Scheduling availability of UE performing measurements in TDD bands	293

9.2A.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH	294

9.2A.6	Intra-frequency measurements with measurement gaps	294

9.2A.6.1	Intra-frequency cell identification	294

9.2A.6.2	Intra-frequency Measurement Period	295

9.2A.7	Intra-frequency RSSI and Channel occupancy measurements	296

9.2A.7.1	Intra-frequency RSSI measurements	296

9.2A.7.2	Intra-frequency Channel occupancy measurements	297

9.2A.7.3	Scheduling restriction during RSSI and Channel Occupancy measurements	298

9.3	NR inter-frequency measurements	298

9.3.1	Introduction	298

9.3.2	Requirements applicability	299

9.3.2.1	Void	299

9.3.2.2	Void	299

9.3.3	Number of cells and number of SSB	299

9.3.3.1	Requirements for FR1	299

9.3.3.2	Requirements for FR2	299

9.3.4	Inter-frequency measurement with measurement gaps	299

9.3.4.1	Void	301

9.3.4.2	Void	301

9.3.5	Inter-frequency measurements	301

9.3.5.1	Void	302

9.3.5.2	Void	302

9.3.5.3	Void	302

9.3.6	Inter-frequency measurements reporting requirements	302

9.3.6.1	Periodic Reporting	302

9.3.6.2	Event-triggered Periodic Reporting	302

9.3.6.3	Event-triggered Reporting	302

9.3.7	Void	303

9.3.8	Inter-frequency SFTD measurement requirements	303

9.3.8.1	Introduction	303

9.3.8.2	SFTD Measurement delay	303

9.3.8.3	SFTD Measurement reporting delay	304

9.3.9	Inter frequency measurements without measurement gaps	304

9.3.9.1	Inter frequency Cell identification	304

9.3.9.2	Measurement period	306

9.3.9.3	Scheduling availability of UE during inter-frequency measurements	306

9.3.9.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	306

9.3.9.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	307

9.3.9.3.3	Scheduling availability of UE performing measurements on FR2	307

9.3.9.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	308

9.3A	NR inter-frequency measurements in carrier frequencies with CCA	308

9.3A.1	Introduction	308

9.3A.2	Requirements applicability	308

9.3A.3	Number of cells and number of SSB	309

9.3A.3.1	Requirements	309

9.3A.4	Inter-frequency cell identification	309

9.3A.5	Inter-frequency measurements	310

9.3A.6	NR Inter-frequency measurements reporting requirements	311

9.3A.6.1	Periodic Reporting	311

9.3A.6.2	Event-triggered Periodic Reporting	311

9.3A.6.3	Event-triggered Reporting	311

9.3A.8	Inter-frequency RSSI measurements	311

9.3A.9	Inter-frequency channel occupancy measurements	312

9.4	Inter-RAT measurements	312

9.4.1	Introduction	312

9.4.2	NR − E-UTRAN FDD measurements	314

9.4.2.1	Introduction	314

9.4.2.2	Requirements when no DRX is used	314

9.4.2.3	Requirements when DRX is used	315

9.4.2.4	Measurement reporting requirements	316

9.4.2.4.1	Periodic Reporting	316

9.4.2.4.2	Event-Triggered Periodic Reporting	316

9.4.2.4.3	Event-Triggered Reporting	316

9.4.3	NR − E-UTRAN TDD measurements	316

9.4.3.1	Introduction	316

9.4.3.2	Requirements when no DRX is used	316

9.4.3.3	Requirements when DRX is used	317

9.4.3.4	Measurement reporting requirements	319

9.4.3.4.1	Periodic Reporting	319

9.4.3.4.2	Event-Triggered Periodic Reporting	319

9.4.3.4.3	Event-Triggered Reporting	319

9.4.4	Inter-RAT RSTD measurements	319

9.4.4.1	NR − E-UTRAN FDD RSTD measurements	319

9.4.4.1.1	Introduction	319

9.4.4.1.2	Requirements	320

9.4.4.2	NR − E-UTRAN TDD RSTD measurements	323

9.4.4.2.1	Introduction	323

9.4.4.2.2	Requirements	324

9.4.5	Inter-RAT E-CID measurements	327

9.4.5.1	NR−E-UTRAN FDD E-CID RSRP and RSRQ measurements	327

9.4.5.1.1	Introduction	327

9.4.5.1.2	Requirements	327

9.4.5.1.3	Measurement Reporting Delay	327

9.4.5.2	NR−E-UTRAN TDD E-CID RSRP and RSRQ measurements	327

9.4.5.2.1	Introduction	327

9.4.5.2.2	Requirements	327

9.4.5.2.3	Measurement Reporting Delay	328

9.4.6	NR − UTRAN FDD measurements	328

9.4.6.1	Introduction	328

9.4.6.2	Requirements when no DRX is used	328

9.4.6.3	Requirements when DRX is used	329

9.4.7	NR – E-UTRAN measurements with autonomous gaps	331

9.4.7.1	CGI identification of an E-UTRA cell with autonomous gaps	331

9.4.7.2	CGI reporting delay	331

9.5	L1-RSRP measurements for Reporting	332

9.5.1	Introduction	332

9.5.2	Requirements applicability	332

9.5.3	Measurement Reporting Requirements	332

9.5.3.1	Periodic Reporting	333

9.5.3.2	Semi-Persistent Reporting	333

9.5.3.3	Aperiodic Reporting	333

9.5.4	L1-RSRP measurement requirements	333

9.5.4.1	SSB based L1-RSRP Reporting	333

9.5.4.2	CSI-RS based L1-RSRP Reporting	335

9.5.4A	Void	338

9.5.4A.1	Void	338

9.5.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	338

9.5.5.1	Measurement restriction for SSB based L1-RSRP	338

9.5.5.2	Measurement restriction for CSI-RS based L1-RSRP	338

9.5.6	Scheduling availability of UE during L1-RSRP measurement	339

9.5.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	339

9.5.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	339

9.5.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	340

9.5.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	340

9.5A	L1-RSRP measurements for Reporting under CCA	341

9.5A.1	Introduction	341

9.5A.2	Requirements applicability	341

9.5A.3	Measurement Reporting Requirements	341

9.5A.3.1	Periodic Reporting	341

9.5A.3.2	Semi-Persistent Reporting	342

9.5A.3.3	Aperiodic Reporting	342

9.5A.4	L1-RSRP measurement requirements	342

9.5A.4.1	SSB based L1-RSRP Reporting	342

9.5A.5	Measurement restriction for L1-RSRP measurement	343

9.5A.5.1	Measurement restriction for SSB based L1-RSRP	343

9.5A.6	Scheduling availability of UE during L1-RSRP measurement	343

9.5A.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH	343

9.5A.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH	344

9.5A.6.3	Scheduling availability of UE performing L1-RSRP measurement in case of FR1-FR2 inter-band CA	344

9.6	NE-DC: Measurements	344

9.6.1	Introduction	344

9.6.2	SFTD Measurements	344

9.6.2.1	Introduction	344

9.6.2.2	SFTD Measurement requirements	344

9.7	Cross Link Interference measurements	345

9.7.1	Introduction	345

9.7.2	SRS-RSRP measurements	345

9.7.2.1	Introduction	345

9.7.2.2	Requirements applicability	345

9.7.2.3	Measurement Reporting Requirements	346

9.7.2.3.1	Periodic Reporting	346

9.7.2.3.2	Event-triggered Periodic Reporting	346

9.7.2.3.3	Event Triggered Reporting	346

9.7.2.4	Measurement capability	346

9.7.2.5	SRS-RSRP measurement period	346

9.7.3	CLI-RSSI measurements	347

9.7.3.1	Introduction	347

9.7.3.2	Requirements applicability	347

9.7.3.3	Measurement Reporting Requirements	347

9.7.3.3.1	Periodic Reporting	347

9.7.3.3.2	Event-triggered Periodic Reporting	347

9.7.3.3.3	Event Triggered Reporting	347

9.7.3.4	Measurement capability	348

9.7.3.5	CLI-RSSI measurement period	348

9.7.4	Scheduling availability of UE during CLI measurements	348

9.7.4.1	Scheduling availability of UE performing measurement on FR1	348

9.7.4.2	Scheduling availability of UE performing measurement on FR2	348

9.8	L1-SINR measurements for Reporting	349

9.8.1	Introduction	349

9.8.2	Requirements applicability	349

9.8.3	Measurement Reporting Requirements	350

9.8.3.1	Periodic Reporting	350

9.8.3.2	Semi-Persistent Reporting	350

9.8.3.3	Aperiodic Reporting	351

9.8.4	L1-SINR measurement requirements	351

9.8.4.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	351

9.8.4.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	354

9.8.5	Measurement restriction for L1-SINR measurement	356

9.8.5.1	Measurement restriction if SSB configured for L1-SINR Measurement	356

9.8.5.2	Measurement restriction if CSI-RS configured for L1-SINR measurement	357

9.8.5.3	Measurement restriction if CSI-IM configured for L1-SINR measurement	357

9.8.6	Scheduling availability of UE during L1-SINR measurement	358

9.8.6.1	Scheduling availability of UE performing L1-SINR measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	358

9.8.6.2	Scheduling availability of UE performing L1-SINR measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	358

9.8.6.4	Scheduling availability of UE performing L1-SINR measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	359

9.9	 NR measurements for positioning	359

9.9.1	Introduction	359

9.9.2	RSTD measurements	360

9.9.2.1	Introduction	360

9.9.2.2	Requirements Applicability	360

9.9.2.3	Measurement Capability	360

9.9.2.4	Measurement Reporting Requirements	360

9.9.2.4.1	Void	360

9.9.2.4.2	Void	360

9.9.2.4.3	Void	360

9.9.2.5	Measurements Period Requirements	360

9.9.2.6	Void	362

9.9.3	PRS-RSRP measurements	362

9.9.3.1	Introduction	362

9.9.3.2	Requirements applicability	362

9.9.3.3	Measurement Capability	362

9.9.3.4	Measurement Reporting Requirements	363

9.9.3.5	Measurement Period Requirements	363

9.9.4	UE Rx-Tx time difference measurements	365

9.9.4.1 Introduction	365

9.9.4.2 Requirements Applicability	365

9.9.4.3 Measurement Capability	365

9.9.4.4 Measurement Reporting Requirements	365

9.9.4.5	Measurement Period Requirements	365

9.9.5	NR E-CID measurements	368

9.9.5.1	Introduction	368

9.9.5.2	Measurement Requirements	368

9.9.5.2.1	Intra-frequency Measurement Requirements	368

9.9.5.2.2	Inter-frequency Measurement Requirements	368

9.9.5.2.3	Measurement Reporting Delay	369

9.10	CSI-RS based L3 measurements	369

9.10.1	Introduction	369

9.10.2	CSI-RS based intra-frequency measurements	369

9.10.2.1	Introduction	369

9.10.2.2	Requirements applicability	370

9.10.2.3	Number of cells and number of CSI-RS	371

9.10.2.3.1	Requirements for FR1	371

9.10.2.3.2	Requirements for FR2	371

9.10.2.4	Measurement Reporting Requirements	371

9.10.2.4.1	Periodic Reporting	371

9.10.2.4.2	Event-triggered Periodic Reporting	371

9.10.2.4.3	Event Triggered Reporting	372

9.10.2.5	Intra-frequency measurements without measurement gaps	372

9.10.2.6	Scheduling availability of UE during CSI-RS based intra-frequency measurements	373

9.10.2.6.1	Scheduling availability of UE performing CSI-RS based measurements in TDD bands	373

9.10.2.6.2	Scheduling availability of UE performing CSI-RS based measurements in FR2	374

9.10.3	CSI-RS based Inter-frequency measurements	374

9.10.3.1	Introduction	374

9.10.3.2	Requirements applicability	374

9.10.3.3	Number of cells and number of CSI-RS resources	375

9.10.3.3.1	Requirements for FR1	375

9.10.3.3.2	Requirements for FR2	375

9.10.3.4	Measurements reporting requirements	375

9.10.3.4.1	Periodic Reporting	375

9.10.3.4.2	Event-triggered Periodic Reporting	375

9.10.3.4.3	Event-triggered Reporting	376

9.10.3.5	Inter frequency measurements with measurement gaps	376

9.11	NR measurements with autonomous gaps	377

9.11.1	Introduction	377

9.11.2	CGI identification of an NR cell with autonomous gaps	377

9.11.3	CGI reporting delay	378

10	Measurement Performance requirements	378

10.1	NR measurements	378

10.1.1	Introduction	378

10.1.2	Intra-frequency RSRP accuracy requirements for FR1	379

10.1.2.1	Intra-frequency SS-RSRP accuracy requirements	379

10.1.2.1.1	Absolute SS-RSRP Accuracy	379

10.1.2.1.2	Relative SS-RSRP Accuracy	380

10.1.2.2	Void	381

10.1.2.3	Intra-frequency CSI-RSRP accuracy requirements	381

10.1.2.3.1	Absolute CSI-RSRP Accuracy	381

10.1.2.3.2	Relative CSI-RSRP Accuracy	382

10.1.2B	Intra-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	383

10.1.2B.1	Intra-frequency SS-RSRP accuracy requirements	383

10.1.2B.1.1	Absolute SS-RSRP Accuracy	383

10.1.3	Intra-frequency RSRP accuracy requirements for FR2	384

10.1.3.1	Intra-frequency SS-RSRP accuracy requirements	384

10.1.3.1.1	Absolute SS-RSRP Accuracy	384

10.1.3.1.2	Relative SS-RSRP Accuracy	385

10.1.3.2	Void	386

10.1.3.3	Intra-frequency CSI-RSRP accuracy requirements	386

10.1.3.3.1	Absolute CSI-RSRP Accuracy	386

10.1.3.3.2	Relative CSI-RSRP Accuracy	386

10.1.3B	Intra-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	387

10.1.3B.1	Intra-frequency SS-RSRP accuracy requirements	387

10.1.3B.1.1	Absolute SS-RSRP Accuracy	388

10.1.4	Inter-frequency RSRP accuracy requirements for FR1	388

10.1.4.1	Inter-frequency SS-RSRP accuracy requirements	388

10.1.4.1.1	Absolute Accuracy of SS-RSRP in FR1	388

10.1.4.1.2	Relative Accuracy of SS-RSRP in FR1	389

10.1.4.2	Void	390

10.1.4.3	Inter-frequency CSI-RSRP accuracy requirements	390

10.1.4.3.1	Absolute Accuracy of CSI-RSRP in FR1	390

10.1.4.3.2	Relative Accuracy of CS-RSRP in FR1	391

10.1.4B	Inter-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	392

10.1.4B.1	Inter-frequency SS-RSRP accuracy requirements	392

10.1.4B.1.1	Absolute Accuracy of SS-RSRP in FR1	392

10.1.5	Inter-frequency RSRP accuracy requirements for FR2	393

10.1.5.1	Inter-frequency SS-RSRP accuracy requirements	393

10.1.5.1.1	Absolute SS-RSRP Accuracy	393

10.1.5.1.2	Relative SS-RSRP Accuracy	394

10.1.5.2	Void	395

10.1.5.3	Inter-frequency CSI-RSRP accuracy requirements	395

10.1.5.3.1	Absolute CSI-RSRP Accuracy	395

10.1.5.3.2	Relative CSI-RSRP Accuracy	395

10.1.5B	Inter-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	396

10.1.6 RSRP Measurement Report Mapping	397

10.1.7	Intra-frequency RSRQ accuracy requirements for FR1	399

10.1.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	399

10.1.7.1.1	Absolute SS-RSRQ Accuracy in FR1	399

10.1.7.2	Intra-frequency CSI-RSRQ accuracy requirements	400

10.1.7.2.1	Absolute CSI-RSRQ Accuracy	400

10.1.7B	Intra-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	401

10.1.7B.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	401

10.1.7B.1.1	Absolute SS-RSRQ Accuracy in FR1	401

10.1.8	Intra-frequency RSRQ accuracy requirements for FR2	402

10.1.8.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	402

10.1.8.1.1	Absolute SS-RSRQ Accuracy in FR2	402

10.1.8.2	Intra-frequency CSI-RSRQ accuracy requirements	403

10.1.8.2.1	Absolute CSI-RSRQ Accuracy	403

10.1.8B	Intra-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	404

10.1.8B.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	404

10.1.8B.1.1	Absolute SS-RSRQ Accuracy in FR2	404

10.1.9	Inter-frequency RSRQ accuracy requirements for FR1	405

10.1.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	405

10.1.9.1.1	Absolute Accuracy of SS-RSRQ in FR1	405

10.1.9.1.2	Relative Accuracy of SS-RSRQ in FR1	406

10.1.9.2	Inter-frequency CSI-RSRQ accuracy requirements	407

10.1.9.2.1	Absolute CSI-RSRQ Accuracy	407

10.1.9.2.2	Relative CSI-RSRQ Accuracy	408

10.1.9B	Inter-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	409

10.1.9B.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	409

10.1.9B.1.1	Absolute Accuracy of SS-RSRQ in FR1	409

10.1.10	Inter-frequency RSRQ accuracy requirements for FR2	410

10.1.10.2	Inter-frequency CSI-RSRQ accuracy requirements	412

10.1.10.2.1	Absolute CSI-RSRQ Accuracy	412

10.1.10.2.2	Relative CSI-RSRQ Accuracy	412

10.1.10B	Inter-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	413

10.1.10B.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	413

10.1.10B.1.1	Absolute Accuracy of SS-RSRQ in FR2	413

10.1.11	RSRQ report mapping	414

10.1.12	Intra-frequency SINR accuracy requirements for FR1	415

10.1.12.2	Intra-frequency CSI-SINR accuracy requirements in FR1	415

10.1.12.2.1	Absolute CSI-SINR Accuracy in FR1	415

10.1.13	Intra-frequency SINR accuracy requirements for FR2	416

10.1.13.2	Intra-frequency CSI-SINR accuracy requirements in FR2	417

10.1.13.2.1	Absolute CSI-SINR Accuracy in FR2	417

10.1.14	Inter-frequency SINR accuracy requirements for FR1	418

10.1.14.2	Inter-frequency CSI-SINR accuracy requirements in FR1	420

10.1.14.2.1	Aboslute Accuracy of CSI-SINR in FR1	420

10.1.15	Inter-frequency SINR accuracy requirements for FR2	422

10.1.15.2	Inter-frequency CSI-SINR accuracy requirements in FR2	424

10.1.15.2.1	Aboslute Accuracy of CSI-SINR in FR2	424

10.1.15.2.2	Relative Accuracy of CSI-SINR in FR2	424

10.1.16	SINR report mapping	425

10.1.17	Power Headroom	426

10.1.18	PCMAX,c,f	427

10.1.19	L1-RSRP accuracy requirements for FR1	427

10.1.20	L1-RSRP accuracy requirements for FR2	431

10.1.21	SFTD accuracy requirements	434

10.1.22	CLI measurement accuracy requirements	438

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	442

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	442

10.1.23	RSTD Measurements	444

10.1.23.1	Introduction	444

10.1.23.2	Measurement Accuracy Requirements	445

10.1.23.3	Report mapping	449

10.1.23.3.1	 Absolute DL RSTD Measurement Reporting	449

10.1.23.3.2	 Differential Reporting for DL RSTD Measurement	451

10.1.23.3.3	Additional Path Report Mapping for DL RSTD	453

10.1.24	PRS-RSRP Measurements	455

10.1.24.1	Introduction	455

10.1.24.2	Measurement Accuracy Requirements	455

10.1.24.2.1 Absolute PRS RSRP accuracy	455

10.1.24.3	Report mapping	459

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	459

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	460

10.1.25	UE Rx-Tx Time Difference Measurements	462

10.1.25.1	Introduction	462

10.1.25.2	Measurement Accuracy Requirements	463

10.1.25.3	Report mapping	468

10.1.25.3.1	 Absolute UE Rx-Tx Measurement Report Mapping	468

10.1.25.3.2	 Differential UE Rx-Tx Measurement Report Mapping	470

10.1.25.3.3	 Additional Path Report Mapping for UE Rx-Tx Time Difference	472

10.1.26	FR2 P-MPR report	474

10.1.26.1	Report mapping	474

10.1.27	L1-SINR accuracy requirements for FR1	474

10.1.27.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured	474

10.1.27.1.1	Absolute Accuracy	474

10.1.27.1.2	Relative Accuracy	475

10.1.27.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured	476

10.1.27.2.1	Absolute Accuracy	476

10.1.27.2.2	Relative Accuracy	477

10.1.27.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated IMR configured	479

10.1.27.3.1	Absolute Accuracy	479

10.1.27.3.2	Relative Accuracy	481

10.1.28	L1-SINR accuracy requirements for FR2	482

10.1.28.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured	482

10.1.28.1.1	Absolute Accuracy	482

10.1.28.1.2	Relative Accuracy	483

10.1.28.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured	484

10.1.28.2.1	Absolute Accuracy	484

10.1.28.2.2	Relative Accuracy	485

10.1.28.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated IMR configured	486

10.1.28.3.1	Absolute Accuracy	486

10.1.28.3.2	Relative Accuracy	487

10.1.29	Intra-frequency RSRQ accuracy requirements under CCA	489

10.1.29.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	489

10.1.29.1.1	Absolute SS-RSRQ Accuracy	489

10.1.30	Inter-frequency RSRQ accuracy requirements under CCA	489

10.1.30.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	489

10.1.30.1.1	Aboslute Accuracy of SS-RSRQ	489

10.1.30.1.2	Relative Accuracy of SS-RSRQ	490

10.1.31	Intra-frequency SINR accuracy requirements under CCA	491

10.1.32	Inter-frequency SINR accuracy requirements under CCA	491

10.1.33	L1-RSRP accuracy requirements under CCA	493

10.1.34	RSSI measurements under CCA	494

10.1.34.1	Intra-frequency absolute RSSI measurement accuracy requirements in FR1	494

10.1.34.2	Inter-frequency absolute RSSI measurement accuracy requirements in FR1	494

10.1.34.3	RSSI measurement report mapping	495

10.1.35	Channel occupancy measurements under CCA	495

10.1.35.1	Intra-frequency channel occupancy measurement accuracy requirements in FR1	495

10.1.35.2	Inter-frequency channel occupancy measurement accuracy requirements in FR1	495

10.1.36	Intra-frequency RSRP accuracy requirements under CCA	496

10.1.36.1	Intra-frequency SS-RSRP accuracy requirements in FR1	496

10.1.36.1.1	Absolute SS-RSRP Accuracy	496

10.1.36.1.2	Relative SS-RSRP Accuracy	496

10.1.37	Inter-frequency RSRP accuracy requirements under CCA	497

10.1.37.1	Inter-frequency SS-RSRP accuracy requirements in FR1	497

10.1.37.1.1	Absolute Accuracy of SS-RSRP	497

10.1.37.1.2	Relative Accuracy of SS-RSRP	498

10.2	E-UTRAN measurements	498

10.2.1	Introduction	498

10.2.2	E-UTRAN RSRP measurements	499

10.2.3	E-UTRAN RSRQ measurements	499

10.2.4	E-UTRAN RSTD measurements	499

10.2.5	E-UTRAN RS-SINR measurements	499

10.2.6	E-UTRAN RSRP measurements for CA/DC Idle Mode Measurements	499

10.2.7	E-UTRAN RSRQ measurements for CA/DC Idle Mode Measurements	500

10.3	UTRAN FDD Measurements	500

10.3.1	UTRAN FDD CPICH RSCP	500

10.3.2	UTRAN FDD CPICH Ec/No	501

10.4	V2X measurements	501

10.4.1	Introduction	501

10.4.2	Intra-frequency PSBCH-RSRP accuracy requirements for FR1	502

10.4.2.1	PSBCH-RSRP Absolute Accuracy	502

10.4.2.2	PSBCH-RSRP Relative Accuracy	502

10.4.3	Intra-Frequency SL-RSSI Measurement Accuracy Requirements for FR1	503

10.4.3.1	Absolute SL-RSSI Accuracy	503

10.4.4	Intra-Frequency L1 SL-RSRP Measurement Accuracy Requirements for FR1	503

10.4.4.1	Absolute L1 SL-RSRP Accuracy	503

11	Void	504

12	V2X Requirements	504

12.1	Introduction	504

12.2	UE Transmit Timing	505

12.2.1	Introduction	505

12.2.2	GNSS as synchronization reference source	505

12.2.3	NR Cell as synchronization reference source	505

12.2.4	E-URTAN Cell as synchronization reference source	506

12.2.5	SyncRef UE as synchronization reference source	506

12.3	Initiation/Cease of SLSS Transmissions	506

12.3.1	Introduction	506

12.3.1.1	Initiation/Cease of SLSS transmissions with NR cell as synchronization reference source	506

12.3.1.2	Initiation/Cease of SLSS transmissions with EUTRAN cell as synchronization reference source	507

12.3.1.3	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	508

12.3.1.4	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	508

12.4	Selection / Reselection of V2X Synchronization Reference Source	509

12.5	L1 SL-RSRP measurements	510

12.5.1	Introduction	510

12.5.2	SL-RSRP measurements	510

12.6	Congestion Control measurements	510

12.7	Interruption	510

12.7.1	Interruptions to WAN due to V2X Sidelink Communication	510

12.7.2	V2X Sidelink Communication Dropping due to synchronization source change	511

12.7.3	Interruptions to WAN due to switching between E-UTRA V2X Sidelink and NR V2X Sidelink	513

12.8	Reliability of GNSS signal	513

12.9	Scheduling availability	513

12.9.1	Scheduling availability of UE switching between E-UTRA sidelink and NR sidelink	513

13	Measurement Performance Requirements for NR gNB	514

13.1	UL-RTOA	514

13.1.1	Report mapping	514

13.2	gNB Rx-Tx time difference	516

13.2.1	Report mapping	516

13.2.2	Measurement Accuracy Requirements	518

13.2.2.1	Introduction	518

13.2.2.2	Requirements	518

13.3	UL SRS RSRP measurement	519

13.3.1	Report mapping	519

13.3.2	Measurement accuracy requirements	520

13.3.2.1	Introduction	520

13.3.2.2	Requirements	520

13.4	AoA/ZoA	521

13.4.1	Report mapping	521

Annex A (normative): Test Cases	423

A.1	Purpose of annex	423

A.2	Requirement classification for statistical testing	423

A.2.1	Types of requirements in TS 38.133	423

A.2.1.1	Time and delay requirements on UE higher layer actions	423

A.2.1.2	Measurements of power levels, relative powers and time	423

A.2.1.3	Implementation requirements	424

A.2.1.4	Physical layer timing requirements	424

A.2.1.5	Requirements under CCA	424

A.3	RRM test configurations	425

A.3.1	Reference measurement channels	425

A.3.1.1	PDSCH	425

A.3.1.1.1	FDD	425

A.3.1.1.2	TDD	426

A.3.1A	Reference measurement channels under CCA	429

A.3.1A.1	PDSCH	429

A.3.1A.1.1	TDD	429

A.3.1A.2	CORESET for RMSI scheduling	430

A.3.1A.2.1	TDD	430

A.3.1A.3	CORESET for RMC scheduling	431

A.3.1A.3.1	TDD	431

A.3.1A.4	TDD UL/DL configuration	432

A.3.1A.5	RMC burst transmission model	432

A.3.1.2	CORESET for RMSI scheduling	433

A.3.1.2.1	FDD	433

A.3.1.2.2	TDD	434

A.3.1.3	CORESET for RMC scheduling	437

A.3.1.3.1	FDD	437

A.3.1.3.2	TDD	438

A.3.1.4	TDD UL/DL configuration	441

A.3.2	OFDMA channel noise generator (OCNG)	442

A.3.2.1	Generic OFDMA Channel Noise Generator (OCNG)	442

A.3.2.1.1	OCNG pattern 1: Generic OCNG pattern for all unused REs	442

A.3.2.1.2	OCNG pattern 2: Generic OCNG pattern for all unused REs for 2AoA setup	443

A.3.2.1.3	OCNG pattern 3: Generic OCNG pattern for unused REs in the same bandwidth as CORESET	443

A.3.2.1.4	OCNG pattern 4: Generic OCNG pattern for all unused REs outside SSB slot(s)	444

A.3.2.2	Void	445

A.3.3	Reference DRX configurations	445

A.3.3.1	DRX Configuration 1: DRX cycle = 40 ms and TAT = 500 ms	445

A.3.3.2	DRX Configuration 2: DRX cycle = 640 ms and TAT = 500 ms	445

A.3.3.3	DRX Configuration 3: DRX cycle = 40 ms and TAT = Infinity	445

A.3.3.4	DRX Configuration 4: DRX cycle = 160 ms and TAT = Infinity	446

A.3.3.5	DRX Configuration 5: DRX cycle = 320 ms and TAT = Infinity	446

A.3.3.6	DRX Configuration 6: DRX cycle = 320 ms and TAT = 500 ms	446

A.3.3.7	DRX Configuration 7: DRX cycle = 640 ms and TAT = Infinity	447

A.3.3.8	DRX Configuration 8: DRX cycle = 320 ms and TAT = Infinity	447

A.3.3.9	DRX Configuration 9: DRX cycle = 40 ms and TAT = 500 ms	447

A.3.3.10	DRX Configuration 10: DRX cycle = 640 ms and TAT = 500 ms	448

A.3.3.11	DRX Configuration 11: DRX cycle = 20 ms and TAT = Infinity	448

A.3.3.12	DRX Configuration 12: DRX cycle = 640 ms and TAT = Infinity	448

A.3.4	Test Cases with Different Channel Bandwidths	448

A.3.4.1	Test Cases with Different E-UTRA Channel Bandwidths	448

A.3.4.1.1	Introduction	448

A.3.4.1.2	Principle of testing	449

A.3.5	Test Cases for Synchronous and Asynchronous DC Operations	449

A.3.5.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations	449

A.3.5.1.1	Introduction	449

A.3.5.1.2	Principle of Testing	449

A.3.6	Antenna configurations	449

A.3.6.1	Antenna configurations for FR1	449

A.3.6.1.1	Antenna connection for 4 Rx capable UEs	449

A.3.6.1.1.1	Introduction	449

A.3.6.1.1.2	Principle of testing	449

A.3.6.2	Antenna configurations for FR2	452

A.3.6A	Antenna configurations with unlicensed bands	452

A.3.6A.1	Antenna configurations for FR1	452

A.3.6A.1.1	Antenna connection for 4 Rx capable UEs	452

A.3.6A.1.1.1	Introduction	452

A.3.6A.1.1.2	Principle of testing	452

A.3.7	EN-DC test setup	454

A.3.7.1	Introduction	454

A.3.7.2	E-UTRAN Serving Cell Parameters	454

A.3.7.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	454

A.3.7.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	455

A.3.7A	NR FR1-FR2 test setup	456

A.3.7B	EN-DC test setup with unlicensed bands	457

A.3.7B.1	Introduction	457

A.3.7B.2	E-UTRAN Serving Cell Parameters	457

A.3.7B.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) under CCA in FR1	457

A.3.7C	LTE-FR1/FR2 test setup	458

A.3.7D	NE-DC test setup	458

A.3.7D.1	Introduction	458

A.3.7D.2	E-UTRAN Serving Cell Parameters	458

A.3.7D.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	458

A.3.7D.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	458

A.3.8	PRACH configurations	458

A.3.8.1	Introduction	458

A.3.8.2	PRACH configurations in FR1	459

A.3.8.2.1	FR1 PRACH configuration 1	459

A.3.8.2.2	FR1 PRACH configuration 2	459

A.3.8.2.3	FR1 PRACH configuration 3	460

A.3.8.2.4	FR1 PRACH configuration 4	461

A.3.8.3	PRACH configurations in FR2	462

A.3.8.3.1	FR2 PRACH configuration 1	462

A.3.8.3.2	FR2 PRACH configuration 2	463

A.3.8.3.3	FR2 PRACH configuration 3	464

A.3.8.3.4	FR2 PRACH configuration 4	465

A.3.8A	PRACH configurations under CCA	466

A.3.8A.1	Introduction	466

A.3.8A.2	PRACH configurations in FR1	466

A.3.8A.2.1	FR1 PRACH configuration 1 under CCA	466

A.3.8A.2.2	FR1 PRACH configuration 2 under CCA	467

A.3.9	BWP configurations	468

A.3.9.1	Introduction	468

A.3.9.2	Downlink BWP configurations	469

A.3.9.2.1	Initial BWP	469

A.3.9.2.2	Dedicated BWP	469

A.3.9.3	Uplink BWP configurations	469

A.3.9.3.1	Initial BWP	469

A.3.9.3.2	Dedicated BWP	470

A.3.10	SSB Configurations	470

A.3.10.1	SSB Configurations for FR1	470

A.3.10.1.1	SSB pattern 1 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	470

A.3.10.1.5	SSB pattern 5 in FR1: SSB allocation for SSB SCS=15 kHz starting from odd SFN in 10 MHz	472

A.3.10A	SSB Configurations under CCA	477

A.3.10A.1	SSB Configurations under CCA for FR1	477

A.3.10A.1.1	SSB pattern 1 under CCA for semi-static channel access: SSB allocation for SSB SCS=30kHz in 40MHz	477

A.3.10A.1.2	SSB pattern 2 under CCA for dynamic channel access: SSB allocation for SSB SCS=30kHz in 40MHz	478

A.3.10A.1.3	SSB pattern 3 under CCA for semi-static channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	478

A.3.10A.1.4	SSB pattern 4 under CCA for dynamic channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	479

A.3.11	SMTC Configurations	479

A.3.11.1	SMTC pattern 1: SMTC period = 20 ms with SMTC duration = 1 ms	479

A.3.11.2	SMTC pattern 2: SMTC period = 20 ms with SMTC duration = 5 ms	479

A.3.11.3	SMTC pattern 3: SMTC period = 160 ms with SMTC duration = 1 ms	480

A.3.11.4	SMTC pattern 4: SMTC period = 20 ms with SMTC duration = 1 ms	480

A.3.11.5	SMTC pattern 5: SMTC period = 20 ms with SMTC duration = 5 ms	480

A.3.11.6	SMTC pattern 6: SMTC period = 20 ms with SMTC duration = 5 ms	480

A.3.12	Test Cases with Different CC Configurations	480

A.3.12.1 EN-DC Test Cases with Different EN-DC Configurations	480

A.3.12.1.1	Introduction	480

A.3.12.1.2	Principle of testing	481

A.3.12.2	Carrier Aggregation Test Cases with Different CA Configurations	481

A.3.12.2.1	Introduction	481

A.3.12.2.2	Principle of testing	481

A.3.13	Test Cases in SA and EN-DC Operations	481

A.3.13.1	Introduction	481

A.3.13.2	Principle of Testing	481

A.3.13A Test Cases involving E-UTRA/FR1 and FR2 carriers	482

A.3.13A.1	Introduction	482

A.3.13A.2	Principle of Testing in EN-DC	482

A.3.13A.3	Principle of Testing in SA	482

A.3.13B	Test Cases for EN-DC and NE-DC Operations	483

A.3.13B.1	Active BWP switch Test Cases for EN-DC and NE-DC Operations	483

A.3.13B.1.1	Introduction	483

A.3.13B.1.2	Principle of Testing	483

A.3.13B.2	SFTD accuracy Test Cases for EN-DC and NE-DC Operations	483

A.3.13B.2.1	Introduction	483

A.3.13B.2.2	Principle of Testing	483

A.3.14	CSI-RS configurations	484

A.3.14.1	FDD	484

A.3.14.2	TDD	486

A.3.15	Angle of Arrival (AoA) for FR2 RRM test cases	493

A.3.15.1	Setup 1: Single AoA in Rx beam peak direction	493

A.3.15.2	Setup 2: Single AoA in non Rx beam peak direction	493

A.3.15.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction	493

A.3.15.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction	494

A.3.15.3	Setup 3: 2 AoAs	494

A.3.15.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak	494

A.3.15.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction	494

A.3.15.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction	494

A.3.16	TCI State Configuration	495

A.3.16.1	Introduction	495

A.3.16.2	TCI states	495

A.3.17	Configurations of CSI-RS for tracking	495

A.3.17.1	Configuration of CSI-RS for tracking for FR1	495

A.3.17.1.1	FDD	495

A.3.17.1.2	TDD	496

A.3.17.2	Configuration of CSI-RS for tracking for FR2	497

A.3.17.2.1	TDD	497

A.3.18	Additional definitions related to OTA testing for FR2 RRM test cases	498

A.3.18.1	Introduction	498

A.3.18.2	PRACH Power Measurement	498

A.3.19	Test applicability for DAPS handover	498

A.3.19.1	Introduction	498

A.3.19.2	Principle of testing	498

A.3.20	MsgA configurations	499

A.3.20.1	Introduction	499

A.3.20.2	MsgA configurations in FR1	499

A.3.20.2.1	FR1 MsgA configuration 1	499

A.3.20.2.2	FR1 MsgA configuration 2	500

A.3.20.3	MsgA configurations in FR2	502

A.3.20.3.1	FR2 MsgA configuration 1	502

A.3.20.3.2	FR2 MsgA configuration 2	503

A.3.20A	MsgA configurations under CCA	505

A.3.20A.1	Introduction	505

A.3.20A.2	MsgA configurations in FR1	505

A.3.20A.2.1	FR1 MsgA configuration 1 under CCA	505

A.3.20A.2.2	FR1 MsgA configuration 2 under CCA	506

A.3.21	V2X sidelink communication	509

A.3.21.1	Introduction	509

A.3.21.2	Reference resource pool configurations for V2X Sidelink Communication	509

A.3.21.3	Reference measurement channels for V2X Sidelink Communication	513

A.3.22	CSI-IM configurations	513

A.3.22.1	FDD	513

A.3.22.2	TDD	514

A.3.23	Spatial Relation Configuration	516

A.3.23.1	Introduction	516

A.3.23.2	Spatial Relation	516

A.3.24	SRS configuration	517

A.3.25	Channel bandwidth (CBW) configurations	519

A.3.25.1	DL UE specific CBW	519

A.3.25.2	UL UE specific CBW	520

A.3.26	CCA model	520

A.3.26.1	Introduction	520

A.3.26.2	CCA model for operation on a carrier frequency with CCA in FR1	520

A.3.26.2.1	DL CCA model	520

A.3.26.2.2	UL CCA model	521

A.3.27	Test Cases with at Least One Cell on a Carrier Frequency with CCA	522

A.3.27.1	Introduction	522

A.3.27.2	 NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	522

A.3.27.3	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	522

A.3.27.4	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	522

A.3.27.5	E-UTRA Standalone Tests with at Least One NR Cell under CCA	523

A.3.28	Discovery Burst Transmission Window configuration under CCA	523

A.3.28.1	DBT Window pattern 1: DBT Window period = 20 ms with DBT Window duration = 1 ms	523

A.3.29	Testing principles for UE capable of only NR bands with shared spectrum access	523

A.3.29.1	Introduction	523

A.3.29.2	Principle of testing for UE capable of EN-DC with only NR bands with shared spectrum access	523

A.3.29.3	Principle of testing for UE capable of SA operation with only NR bands with shared spectrum access	524

A.3.30	CSI-RS configurations for RRM	524

A.3.30.1	FDD	524

A.3.30.2	TDD	525

A.3.31	PRS Configurations	527

A.3.31.1.	PRS Configurations for FR1	527

A.3.31.1.1.	PRS pattern 1 in FR1: SCS=15 KHz	527

A.3.31.1.2.	PRS pattern 2 in FR1: SCS=30 KHz	528

A.3.31.2.	PRS Configurations for FR2	528

A.3.31.2.1.	PRS pattern 1 in FR2: SCS=120 KHz	528

A.4	EN-DC tests with all NR cells in FR1	529

A.4.1	Void	529

A.4.2	Void	529

A.4.3	RRC_CONNECTED state mobility	529

A.4.3.1	Void	529

A.4.3.2	RRC Connection Mobility Control	529

A.4.3.2.1	Void	529

A.4.3.2.2	Random Access	529

A.4.3.2.2.1	4-step RA type contention based random access test in FR1 for PSCell in EN-DC	529

A.4.3.2.2.2	4-step RA type non-contention based random access test in FR1 for PSCell in EN-DC	533

A.4.3.2.2.3	2-step RA type contention based random access test in FR1 for PSCell in EN-DC	537

A.4.3.2.2.4	2-step RA type non-contention based random access test in FR1 for PSCell in EN-DC	541

A.4.3.2.3	Void	545

A.4.4	Timing	545

A.4.4.1	UE transmit timing	545

A.4.4.1.1	NR UE Transmit Timing Test for FR1	545

A.4.4.1.1.1	Test Purpose and environment	545

A.4.4.1.1.2	Test requirements	549

A.4.4.2	UE timer accuracy	549

A.4.4.3	Timing advance	549

A.4.4.3.1	EN-DC FR1 timing advance adjustment accuracy	549

A.4.4.3.1.1	Test Purpose and Environment	549

A.4.4.3.1.2	Test Parameters	549

A.4.4.3.1.3	Test Requirements	553

A.4.5	Signaling characteristics	553

A.4.5.1	Radio link Monitoring	553

A.4.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	554

A.4.5.1.1.1	Test Purpose and Environment	554

A.4.5.1.1.2	Test Requirements	558

A.4.5.1.2	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	558

A.4.5.1.2.1	Test Purpose and Environment	558

A.4.5.1.2.2	Test Requirements	564

A.4.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	564

A.4.5.1.3.1	Test Purpose and Environment	564

A.4.5.1.3.2	Test Requirements	570

A.4.5.1.4	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	570

A.4.5.1.4.1	Test Purpose and Environment	570

A.4.5.1.4.2	Test Requirements	576

A.4.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	576

A.4.5.1.5.1	Test Purpose and Environment	576

A.4.5.1.5.2	Test Requirements	582

A.4.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	582

A.4.5.1.6.1	Test Purpose and Environment	582

A.4.5.1.6.2	Test Requirements	588

A.4.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	588

A.4.5.1.7.1	Test Purpose and Environment	588

A.4.5.1.7.2	Test Requirements	593

A.4.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	594

A.4.5.1.8.1	Test Purpose and Environment	594

A.4.5.1.8.2	Test Requirements	599

A.4.5.2	Interruption	599

A.4.5.2.1	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	599

A.4.5.2.1.1	Test Purpose and Environment	599

A.4.5.2.1.2	Test Requirements	603

A.4.5.2.2	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	603

A.4.5.2.2.1	Test Purpose and Environment	603

A.4.5.2.2.2	Test Requirements	607

A.4.5.2.3	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	607

A.4.5.2.3.1	Test Purpose and Environment	607

A.4.5.2.3.2	Test Requirements	611

A.4.5.2.4	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	612

A.4.5.2.4.1	Test Purpose and Environment	612

A.4.5.2.4.2	Test Requirements	616

A.4.5.2.5	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	617

A.4.5.2.5.1	Test Purpose and Environment	617

A.4.5.2.5.2	Test Requirements	620

A.4.5.2.6	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	620

A.4.5.2.6.1	Test Purpose and Environment	620

A.4.5.2.6.2	Test Requirements	624

A.4.5.2.7	Void	624

A.4.5.2.8	E-UTRAN - NR FR1 interruptions at NR SRS carrier based switching in asynchronous EN-DC	624

A.4.5.2.8.1	Test Purpose and Environment	624

A.4.5.2.8.2	Test Requirements	627

A.4.5.2.9	E-UTRAN – NR interruptions at E-UTRA SRS carrier based switching	628

A.4.5.2.9.1	Test Purpose and Environment	628

A.4.5.2.9.2	Test Requirements	632

A.4.5.3	SCell Activation and Deactivation Delay	633

A.4.5.3.1	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	633

A.4.5.3.1.1	Test Purpose and Environment	633

A.4.5.3.1.2	Test Requirements	638

A.4.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 640 ms SCell measurement cycle	639

A.4.5.3.2.1	Test Purpose and Environment	639

A.4.5.3.2.2	Test Requirements	639

A.4.5.3.3	SCell Activation and deactivation of unknown SCell in FR1	639

A.4.5.3.3.1	Test Purpose and Environment	639

A.4.5.3.3.2	Test Requirements	640

A.4.5.3.4	SCell Activation and deactivation of multiple unknown SCells in FR1 with single activation/deactivation command	640

A.4.5.3.4.1	Test Purpose and Environment	640

A.4.5.3.4.2	Test Requirements	645

A.4.5.3.5	Direct SCell activation at SCell addition of known SCell in FR1	645

A.4.5.3.5.1	Test Purpose and Environment	645

A.4.5.3.5.2	Test Requirements	649

A.4.5.4	UE UL carrier RRC reconfiguration Delay	649

A.4.5.4.1	UE UL carrier RRC reconfiguration Delay	649

A.4.5.4.1.1 Test Purpose and Environment	649

A.4.5.4.1.2	Test Requirements	657

A.4.5.5	Beam Failure Detection and Link recovery procedures	658

A.4.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	658

A.4.5.5.1.1	Test Purpose and Environment	658

A.4.5.5.1.2	Test Requirements	663

A.4.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	663

A.4.5.5.2.1	Test Purpose and Environment	663

A.4.5.5.2.2	Test Requirements	669

A.4.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	669

A.4.5.5.3.1	Test Purpose and Environment	669

A.4.5.5.3.2	Test Requirements	675

A.4.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in DRX mode	675

A.4.5.5.4.1	Test Purpose and Environment	675

A.4.5.5.4.2	Test Requirements	681

A.4.5.5.5	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	681

A.4.5.5.5.1	Test Purpose and Environment	681

A.4.5.5.5.2	Test Requirements	687

A.4.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	687

A.4.5.5.6.1	Test Purpose and Environment	687

A.4.5.5.6.2	Test Requirements	694

A.4.5.6	Active BWP switch	695

A.4.5.6.1	DCI-based and Timer-based Active BWP Switch	695

A.4.5.6.1.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	695

A.4.5.6.1.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	700

A.4.5.6.2	RRC-based Active BWP Switch	705

A.4.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	709

A.4.5.6.3.1	Simultaneous E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in EN-DC on multiple CCs	709

A.4.5.6.3.1.1	Test Purpose and Environment	709

A.4.5.6.3.1.2	Test Requirements	714

A.4.5.6.4	SCell dormancy switch	715

A.4.5.6.4.1	E-UTRAN – NR FR1 PSCell SCell dormancy switch of single FR1 SCell outside active time	715

A.4.5.6.4.1.1	Test Purpose and Environment	715

A.4.5.6.4.1.2	Test Requirements	720

A.4.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR1 SCells inside active time	720

A.4.5.6.4.2.1	Test Purpose and Environment	720

A.4.5.6.4.2.2	Test Requirements	725

A.4.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	725

A.4.5.6.5.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC on multiple CCs	725

A.4.5.6.5.1.1	Test Purpose and Environment	725

A.4.5.6.5.1.2	Test Requirements	730

A.4.5.7	PSCell addition and release delay	730

A.4.5.7.1	Addition and Release Delay of known NR PSCell	730

A.4.5.7.1.1	Test purpose and environment	730

A.4.5.7.1.2	Test Requirements	734

A.4.5.8	DL Interruptions at switching between two uplink carriers	735

A.4.5.8.1	Test Purpose and Environment	735

A.4.5.8.2	Test Requirements	738

A.4.5.9	UE specific CBW change	739

A.4.5.9.1	UE specific CBW change on FR1 NR PSCell with non-DRX in synchronous EN- DC	739

A.4.5.9.1.1	Test Purpose and Environment	739

A.4.5.9.1.2	Test Requirements	743

A.4.6	Measurement procedure	743

A.4.6.1	Intra-frequency Measurements	743

A.4.6.1.1	EN-DC event triggered reporting tests without gap under non-DRX	743

A.4.6.1.1.1	Test purpose and Environment	743

A.4.6.1.1.2	Test parameters	743

A.4.6.1.1.3	Test Requirements	747

A.4.6.1.2	EN-DC event triggered reporting tests without gap under DRX	747

A.4.6.1.2.1	Test purpose and Environment	747

A.4.6.1.2.2	Test parameters	747

A.4.6.1.2.2	Test Requirements	751

A.4.6.1.3	EN-DC event triggered reporting tests with per-UE gaps under non-DRX	751

A.4.6.1.3.1	Test purpose and Environment	751

A.4.6.1.3.2	Test parameters	751

A.4.6.1.3.3	Test Requirements	755

A.4.6.1.4	EN-DC event triggered reporting tests with per-UE gaps under DRX	755

A.4.6.1.4.1	Test purpose and Environment	755

A.4.6.1.4.2	Test parameters	755

A.4.6.1.4.3	Test Requirements	759

A.4.6.1.5	EN-DC event triggered reporting tests without gap under non-DRX with SSB index reading	759

A.4.6.1.5.1	Test purpose and Environment	759

A.4.6.1.5.2	Test parameters	759

A.4.6.1.5.3	Test Requirements	761

A.4.6.1.6	EN-DC event triggered reporting tests with SSB index reading with per-UE gaps	762

A.4.6.1.6.1	Test purpose and Environment	762

A.4.6.1.6.2	Test parameters	762

A.4.6.1.6.3	Test Requirements	764

A.4.6.1.7	EN-DC event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	765

A.4.6.1.7.1	Test purpose and Environment	765

A.4.6.1.7.2	Test parameters	765

A.4.6.1.7.3	Test Requirements	769

A.4.6.2	Inter-frequency Measurements	769

A.4.6.2.1	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	769

A.4.6.2.1.1	Test Purpose and Environment	769

A.4.6.2.1.2	Test Requirements	773

A.4.6.2.2	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	773

A.4.6.2.2.1	Test Purpose and Environment	773

A.4.6.2.2.2	Test Requirements	777

A.4.6.2.3	Void	778

A.4.6.2.4	Void	778

A.4.6.2.5	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	778

A.4.6.2.5.1	Test Purpose and Environment	778

A.4.6.2.5.2	Test Requirements	782

A.4.6.2.6	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	783

A.4.6.2.6.1	Test Purpose and Environment	783

A.4.6.2.6.2	Test Requirements	787

A.4.6.2.7	Void	788

A.4.6.2.8	Void	788

A.4.6.3	Void	788

A.4.6.4	L1-RSRP measurement for beam reporting	788

A.4.6.4.1	SSB based L1-RSRP measurement when DRX is not used	788

A.4.6.4.1.1	Test Purpose and Environment	788

A.4.6.4.1.2	Test parameters	788

A.4.6.4.1.3	Test Requirements	791

A.4.6.4.2	SSB based L1-RSRP measurement when DRX is used	791

A.4.6.4.2.1	Test Purpose and Environment	791

A.4.6.4.2.2	Test parameters	792

A.4.6.4.2.3	Test Requirements	795

A.4.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	795

A.4.6.4.3.1	Test Purpose and Environment	795

A.4.6.4.3.2	Test parameters	796

A.4.6.4.3.3	Test Requirements	799

A.4.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	799

A.4.6.4.4.1	Test Purpose and Environment	799

A.4.6.4.4.2	Test parameters	800

A.4.6.4.4.3	Test Requirements	802

A.4.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	802

A.4.6.4.5.1	Test Purpose and Environment	802

A.4.6.4.5.2	Test parameters	803

A.4.6.4.5.3	Test Requirements	806

A.4.6.5	CLI measurements	806

A.4.6.5.1	SRS-RSRP measurement with non-DRX	806

A.4.6.5.1.1	Test Purpose and Environment	806

A.4.6.5.1.2	Test Parameters	807

A.4.6.5.1.3	Test Requirements	810

A.4.6.5.2	CLI-RSSI measurement with non-DRX	810

A.4.6.5.2.1	Test Purpose and Environment	810

A.4.6.5.2.2	Test Parameters	811

A.4.6.5.2.3	Test Requirements	812

A.4.6.6	Measurements with autonomous gaps	813

A.4.6.6.1	EN-DC intra-frequency CGI identification of NR FR1 cell with autonomous gaps in synchronous EN-DC	813

A.4.6.6.1.1	Test Purpose and Environment	813

A.4.6.6.1.2	Test Requirements	817

A.4.6.7	L1-SINR measurement for beam reporting	817

A.4.6.7.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR when DRX is not used	817

A.4.6.7.1.1	Test Purpose and Environment	817

A.4.6.7.1.2	Test parameters	818

A.4.6.7.1.3	Test Requirements	821

A.4.6.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	821

A.4.6.7.2.1	Test Purpose and Environment	821

A.4.6.7.2.2	Test parameters	822

A.4.6.7.2.3	Test Requirements	825

A.4.6.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	825

A.4.6.7.3.1	Test Purpose and Environment	825

A.4.6.7.3.2	Test parameters	826

A.4.6.7.3.3	Test Requirements	829

A.4.6.8	CSI-RS based intra-frequency Measurement	829

A.4.6.8.1	EN-DC event triggered reporting tests without gap under DRX	829

A.4.6.8.1.1	Test purpose and Environment	829

A.4.6.8.1.2	Test Requirements	832

A.4.6.9	CSI-RS based inter-frequency Measurement	833

A.4.6.9.1	EN-DC event triggered reporting tests for FR1 cell when non-DRX is used	833

A.4.6.9.1.1	Test Purpose and Environment	833

A.4.6.9.1.2	Test Requirements	836

A.4.7	Measurement Performance requirements	837

A.4.7.1	SS-RSRP	837

A.4.7.1.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	837

A.4.7.1.1.1	Test Purpose and Environment	837

A.4.7.1.1.2	Test parameters	837

A.4.7.1.1.3	Test Requirements	841

A.4.7.1.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	841

A.4.7.1.2.1	Test Purpose and Environment	841

A.4.7.1.2.2	Test parameters	842

A.4.7.1.2.3	Test Requirements	846

A.4.7.1.3	Void	846

A.4.7.2	SS-RSRQ	846

A.4.7.2.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	846

A.4.7.2.1.1	Test Purpose and Environment	846

A.4.7.2.1.2	Test Parameters	847

A.4.7.2.1.3	Test Requirements	852

A.4.7.2.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	852

A.4.7.2.2.1	Test Purpose and Environment	852

A.4.7.2.2.2	Test Parameters	852

A.4.7.2.2.3	Test Requirements	857

A.4.7.3	SS-SINR	857

A.4.7.3.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	857

A.4.7.3.1.1	Test Purpose and Environment	857

A.4.7.3.1.2	Test Parameters	857

A.4.7.3.1.3	Test Requirements	861

A.4.7.3.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	861

A.4.7.3.2.1	Test Purpose and Environment	861

A.4.7.3.2.2	Test Parameters	861

A.4.7.3.2.3	Test Requirements	866

A.4.7.4	L1-RSRP measurement for beam reporting	866

A.4.7.4.1	SSB based L1-RSRP measurement	866

A.4.7.4.1.1	Test Purpose and Environment	866

A.4.7.4.1.2	Test parameters	867

A.4.7.4.1.3	Test Requirements	870

A.4.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	870

A.4.7.4.2.1	Test Purpose and Environment	870

A.4.7.4.2.2	Test parameters	871

A.4.7.4.2.3	Test Requirements	874

A.4.7.5	SFTD accuracy	874

A.4.7.5.1	SFTD accuracy	874

A.4.7.5.1.1	Test Purpose and Environment	874

A.4.7.5.1.2	Test Parameters	874

A.4.7.5.1.3	Test Requirements	879

A.4.7.5.2	Void	879

A.4.7.5.3	Void	879

A.4.7.6	CLI measurements	879

A.4.7.6.1	EN-DC SRS-RSRP measurement accuracy with FR1 serving cell	879

A.4.7.6.1.1	Test Purpose and Environment	879

A.4.7.6.1.2	Test parameters	880

A.4.7.6.1.3	Test Requirements	885

A.4.7.6.2	EN-DC CLI-RSSI measurement accuracy with FR1 serving cell	885

A.4.7.6.2.1	Test Purpose and Environment	885

A.4.7.6.2.2	Test parameters	886

A.4.7.6.2.3	Test Requirements	889

A.4.7.7	L1-SINR measurement for beam reporting	889

A.4.7.7.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	889

A.4.7.7.1.1	Test Purpose and Environment	889

A.4.7.7.1.2	Test parameters	890

A.4.7.7.1.3	Test Requirements	894

A.4.7.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR	894

A.4.7.7.2.1	Test Purpose and Environment	894

A.4.7.7.2.2	Test parameters	895

A.4.7.7.2.3	Test Requirements	899

A.4.7.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	899

A.4.7.7.3.1	Test Purpose and Environment	899

A.4.7.7.3.2	Test parameters	900

A.4.7.7.3.3	Test Requirements	903

A.4.7.8	CSI-RSRP	903

A.4.7.8.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	903

A.4.7.8.1.1	Test Purpose and Environment	903

A.4.7.8.1.2	Test parameters	904

A.4.7.8.1.3	Test Requirements	907

A.4.7.8.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	907

A.4.7.8.2.1	Test Purpose and Environment	907

A.4.7.8.2.2	Test parameters	908

A.4.7.8.2.3	Test Requirements	911

A.4.7.9	CSI-RSRQ	912

A.4.7.9.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	912

A.4.7.9.1.1	Test Purpose and Environment	912

A.4.7.9.1.2	Test Parameters	912

A.4.7.9.1.3	Test Requirements	917

A.4.7.9.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	917

A.4.7.9.2.1	Test Purpose and Environment	917

A.4.7.9.2.2	Test Parameters	917

A.4.7.2.2.3	Test Requirements	922

A.4.7.10	CSI-SINR	922

A.4.7.10.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	922

A.4.7.10.1.1	Test Purpose and Environment	922

A.4.7.10.1.2	Test Parameters	923

A.4.7.10.1.3	Test Requirements	926

A.4.7.10.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	926

A.4.7.10.2.1	Test Purpose and Environment	926

A.4.7.10.2.2	Test Parameters	926

A.4.7.10.2.3	Test Requirements	929

A.4.8	Void	930

A.4A	NE-DC test with all NR cells in FR1	930

A.4A.1	Signaling characteristics	930

A.4A.1.1	E-UTRAN PSCell addition	930

A.4A.1.1.1	Test purpose and environment	930

A.4A.1.1.2	Test Requirements	934

A.4A.1.2	Active BWP switch	935

A.4A.1.2.1	E-UTRAN PSCell – NR PCell FR1 DCI-based and Timer-based DL active BWP switch in non-DRX in synchronous NE-DC	935

A.4A.1.2.1.1	Test Purpose and Environment	935

A.4A.1.2.1.2	Test Requirements	939

A.4A.2	Measurement performance	940

A.4A.2.1	SFTD accuracy	940

A.4A.2.1.1	SFTD accuracy	940

A.4A.2.1.1.1	Test Purpose	940

A.4A.2.1.1.2	Test Environment	940

A.4A.2.1.1.3	Test Requirements	943

A.5	EN-DC tests with one or more NR cells in FR2	944

A.5.1	Void	944

A.5.2	Void	944

A.5.3	RRC_CONNECTED state mobility	944

A.5.3.1	Void	944

A.5.3.2	RRC Connection Mobility Control	944

A.5.3.2.1	Void	944

A.5.3.2.2	Random Access	944

A.5.3.2.2.1	4-step RA type contention based random access test in FR2 for PSCell/SCell in EN-DC	944

A.5.3.2.2.2	4-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	947

A.5.3.2.2.3	2-step RA type contention based random access test in FR2 for PSCell/SCell in EN-DC	953

A.5.3.2.2.4	2-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	956

A.5.3.2.3	Void	960

A.5.4	Timing	960

A.5.4.1	UE transmit timing	960

A.5.4.1.1	NR UE Transmit Timing Test for FR2	960

A.5.4.1.1.1	Test Purpose and environment	960

A.5.4.1.1.2	Test requirements	963

A.5.4.2	UE timer accuracy	964

A.5.4.3	Timing advance	964

A.5.4.3.1 EN-DC FR2 timing advance adjustment accuracy	964

A.5.4.3.1.1 Test Purpose and Environment	964

A.5.4.3.1.2 Test Parameters	964

A.5.4.3.1.3	Test Requirements	968

A.5.5	Signaling characteristics	968

A.5.5.1	Radio link Monitoring	968

A.5.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	968

A.5.5.1.1.1	Test Purpose and Environment	968

A.5.5.1.1.2	Test Requirements	972

A.5.5.1.2	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	973

A.5.5.1.2.1	Test Purpose and Environment	973

A.5.5.1.2.2	Test Requirements	976

A.5.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	977

A.5.5.1.3.1	Test Purpose and Environment	977

A.5.5.1.3.2	Test Requirements	981

A.5.5.1.4	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	981

A.5.5.1.4.1	Test Purpose and Environment	981

A.5.5.1.4.2	Test Requirements	985

A.5.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	985

A.5.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	989

A.5.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	993

A.5.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	998

A.5.5.1.8.2	Test Requirements	1002

A.5.5.1.9	EN-DC Radio Link Monitoring UE Scheduling Restrictions on FR2	1002

A.5.5.1.9.1	Test Purpose and Environment	1002

A.5.5.1.9.2	Test Requirements	1005

A.5.5.2	Interruption	1005

A.5.5.2.1	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	1005

A.5.5.2.1.1	Test Purpose and Environment	1005

A.5.5.2.1.2	Test Requirements	1008

A.5.5.2.2	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	1008

A.5.5.2.2.1	Test Purpose and Environment	1008

A.5.5.2.2.2	Test Requirements	1011

A.5.5.2.3	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	1011

A.5.5.2.3.1	Test Purpose and Environment	1011

A.5.5.2.3.2	Test Requirements	1015

A.5.5.2.4	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	1015

A.5.5.2.4.1	Test Purpose and Environment	1015

A.5.5.2.4.2	Test Requirements	1018

A.5.5.2.5	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	1019

A.5.5.2.5.1	Test Purpose and Environment	1019

A.5.5.2.5.2	Test Requirements	1022

A.5.5.2.6	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	1023

A.5.5.2.6.1	Test Purpose and Environment	1023

A.5.5.2.6.2	Test Requirements	1025

A.5.5.2.7	E-UTRAN – NR FR2 interruptions at E-UTRA SRS carrier based switching	1026

A.5.5.2.7.1	Test Purpose and Environment	1026

A.5.5.2.7.2	Test Requirements	1029

A.5.5.2.8 E-UTRAN – NR FR2 interruptions at NR SRS carrier based switching	1029

A.5.5.2.8.1 Test Purpose and Environment	1029

A.5.5.2.8.3	Test Requirements	1031

A.5.5.3	SCell Activation and Deactivation Delay	1031

A.5.5.3.1	SCell Activation and deactivation of SCell in FR2 intra-band	1031

A.5.5.3.1.1	Test Purpose and Environment	1031

A.5.5.3.1.2	Test Requirements	1034

A.5.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	1034

A.5.5.3.2.1	Test Purpose and Environment	1034

A.5.5.3.2.2	Test Requirements	1038

A.5.5.3.3	Void	1038

A.5.5.3.4	Void	1038

A.5.5.3.5	SCell Activation and deactivation of SCell in FR2	1038

A.5.5.3.5.1	Test Purpose and Environment	1038

A.5.5.3.5.2	Test Requirements	1041

A.5.5.3.6	Multiple SCell Activation and deactivation of one unknown SCell and one known SCell in FR2	1042

A.5.5.3.6.1	Test Purpose and Environment	1042

A.5.5.3.6.2	Test Requirements	1046

A.5.5.3.7	Direct SCell activation at SCell addition of known SCell in FR2	1047

A.5.5.3.7.1	Test Purpose and Environment	1047

A.5.5.3.7.2	Test Requirements	1050

A.5.5.4	Void	1050

A.5.5.5	Beam Failure Detection and Link recovery procedures	1050

A.5.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1050

A.5.5.5.1.1	Test Purpose and Environment	1050

A.5.5.5.1.2	Test Requirements	1054

A.5.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode	1054

A.5.5.5.2.1	Test Purpose and Environment	1054

A.5.5.5.2.2	Test Requirements	1058

A.5.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	1058

A.5.5.5.3.1	Test Purpose and Environment	1058

A.5.5.5.3.2	Test Requirements	1062

A.5.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1062

A.5.5.5.4.1	Test Purpose and Environment	1062

A.5.5.5.4.2	Test Requirements	1066

A.5.5.5.5	EN-DC scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1066

A.5.5.5.5.1	Test Purpose and Environment	1066

A.5.5.5.5.2	Test Requirements	1070

A.5.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	1070

A.5.5.5.6.1	Test Purpose and Environment	1070

A.5.5.5.6.2	Test Requirements	1074

A.5.5.5.7	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	1074

A.5.5.5.7.1	Test Purpose and Environment	1074

A.5.5.5.7.2	Test Requirements	1078

A.5.5.6	Active BWP switch	1078

A.5.5.6.1	DCI-based and Timer-based Active BWP Switch	1078

A.5.5.6.1.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	1078

A.5.5.6.1.1.1	Test Purpose and Environment	1078

A.5.5.6.1.1.2	Test Requirements	1082

A.5.5.6.1.2	E-UTRAN – NR PSCell FR2 with FR2 SCell DL active BWP switch in non-DRX in synchronous EN-DC	1082

A.5.5.6.2	RRC-based Active BWP Switch	1087

A.5.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1091

A.5.5.6.3.1	E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs in synchronous EN-DC	1091

A.5.5.6.3.1.1	Test Purpose and Environment	1091

A.5.5.6.3.1.2	Test Requirements	1095

A.5.5.6.4	SCell dormancy switch	1095

A.5.5.6.4.1	E-UTRAN – NR FR2 PSCell SCell dormancy switch of single FR2 SCell inside active time	1095

A.5.5.6.4.1.1	Test Purpose and Environment	1095

A.5.5.6.4.1.2	Test Requirements	1099

A.5.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR2 SCells outside active time	1099

A.5.5.6.4.2.1	Test Purpose and Environment	1099

A.5.5.6.4.2.2	Test Requirements	1106

A.5.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1106

A.5.5.7	PSCell addition and release delay	1109

A.5.5.7.1	Addition and Release Delay of NR PSCell	1109

A.5.5.7.1.1	Test purpose and environment	1109

A.5.5.7.1.2	Test Requirements	1113

A.5.5.8	Active TCI state switch delay	1114

A.5.5.8.1	MAC-CE based active TCI state switch	1114

A.5.5.8.1.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1114

A.5.5.8.1.1.1	Test Purpose and Environment	1114

A.5.5.8.1.1.2	Test Requirements	1117

A.5.5.8.2	RRC based active TCI state switch	1117

A.5.5.8.2.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1117

A.5.5.8.2.1.1	Test Purpose and Environment	1117

A.5.5.8.2.1.2	Test Requirements	1121

A.5.5.9	Uplink spatial relation switch delay	1121

A.5.5.9.1	MAC-CE based uplink spatial relation switch	1121

A.5.5.9.1.1	E-UTRAN – NR PSCell FR2 uplink spatial relation switch for a known spatial relation	1121

A.5.5.9.1.1.1	Test Purpose and Environment	1121

A.5.5.9.1.1.2	Test Requirements	1124

A.5.5.9.2	RRC based spatial relation switch	1124

A.5.5.9.2.1	E-UTRAN – NR PSCell FR2 spatial relation switch associated with a known DL-RS	1124

A.5.5.9.2.1.1	Test Purpose and Environment	1124

A.5.5.9.2.1.2	Test Requirements	1127

A.5.5.10	UE specific CBW change	1127

A.5.5.10.1	UE specific CBW change on FR2 NR PSCell	1127

A.5.5.10.1.1	Test Purpose and Environment	1127

A.5.5.10.1.2	Test Requirements	1130

A.5.6	Measurement procedure	1131

A.5.6.1	Intra-frequency Measurements	1131

A.5.6.1.1	EN-DC event triggered reporting test without gap under non-DRX	1131

A.5.6.1.1.1	Test purpose and Environment	1131

A.5.6.1.1.2	Test Requirements	1134

A.5.6.1.2	EN-DC event triggered reporting test without gap under DRX	1134

A.5.6.1.2.1	Test purpose and Environment	1134

A.5.6.1.2.2	Test Requirements	1136

A.5.6.1.3	EN-DC event triggered reporting test with per-UE gaps under non-DRX	1137

A.5.6.1.3.1	Test purpose and Environment	1137

A.5.6.1.3.2	Test Requirements	1141

A.5.6.1.4	EN-DC event triggered reporting test with per-UE gaps under DRX	1141

A.5.6.1.4.1	Test purpose and Environment	1141

A.5.6.1.4.2	Test Requirements	1144

A.5.6.2	Inter-frequency Measurements	1145

A.5.6.2.1 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	1145

A.5.6.2.1.1	Test Purpose and Environment	1145

A.5.6.2.1.2	Test Requirements	1148

A.5.6.2.2 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	1148

A.5.6.2.2.1	Test Purpose and Environment	1148

A.5.6.2.2.2	Test Requirements	1152

A.5.6.2.3 	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	1152

A.5.6.2.3.1	Test Purpose and Environment	1152

A.5.6.2.3.2	Test Requirements	1156

A.5.6.2.4	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	1156

A.5.6.2.4.1	Test Purpose and Environment	1156

A.5.6.2.4.2	Test Requirements	1160

A.5.6.2.5	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	1160

A.5.6.2.5.1	Test Purpose and Environment	1160

A.5.6.2.5.2	Test Requirements	1165

A.5.6.2.6	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	1165

A.5.6.2.6.1	Test Purpose and Environment	1165

A.5.6.2.6.2	Test Requirements	1169

A.5.6.2.7	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	1169

A.5.6.2.7.1	Test Purpose and Environment	1169

A.5.6.2.7.2	Test Requirements	1174

A.5.6.2.8	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	1174

A.5.6.2.8.1	Test Purpose and Environment	1174

A.5.6.2.8.2	Test Requirements	1179

A.5.6.3	L1-RSRP measurement for beam reporting	1179

A.5.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1179

A.5.6.3.1.1	Test Purpose and Environment	1179

A.5.6.3.1.2	Test parameters	1179

A.5.6.3.1.3	Test Requirements	1181

A.5.6.3.2	SSB based L1-RSRP measurement when DRX is used	1181

A.5.6.3.2.1	Test Purpose and Environment	1181

A.5.6.3.2.2	Test parameters	1182

A.5.6.3.2.3	Test Requirements	1184

A.5.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	1184

A.5.6.3.3.1	Test Purpose and Environment	1184

A.5.6.3.3.2	Test parameters	1185

A.5.6.3.3.3	Test Requirements	1187

A.5.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	1188

A.5.6.3.4.1	Test Purpose and Environment	1188

A.5.6.3.4.2	Test parameters	1188

A.5.6.3.4.3	Test Requirements	1190

A.5.6.4	CLI measurements	1191

A.5.6.4.1	SRS-RSRP measurement with DRX	1191

A.5.6.4.1.1	Test Purpose and Environment	1191

A.5.6.4.1.2	Test Parameters	1191

A.5.6.4.1.3	Test Requirements	1193

A.5.6.4.2	CLI-RSSI measurement with DRX	1193

A.5.6.4.2.1	Test Purpose and Environment	1193

A.5.6.4.2.2	Test Parameters	1194

A.5.6.4.2.3	Test Requirements	1196

A.5.6.5	Measurements with autonomous gaps	1196

A.5.6.5.1 	EN-DC inter-frequency CGI identification of NR neighbor cell in FR2	1196

A.5.6.5.1.1	Test Purpose and Environment	1196

A.5.6.5.1.2	Test Requirements	1199

A.5.6.6	L1-SINR measurement for beam reporting	1200

A.5.6.6.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is used	1200

A.5.6.6.1.1	Test Purpose and Environment	1200

A.5.6.6.1.2	Test parameters	1200

A.5.6.6.1.3	Test Requirements	1202

A.5.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	1202

A.5.6.6.2.1	Test Purpose and Environment	1202

A.5.6.6.2.2	Test parameters	1203

A.5.6.6.2.3	Test Requirements	1206

A.5.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	1206

A.5.6.6.3.1	Test Purpose and Environment	1206

A.5.6.6.3.2	Test parameters	1207

A.5.6.6.3.3	Test Requirements	1209

A.5.6.7	CSI-RS based Intra-frequency Measurements	1209

A.5.6.7.1	EN-DC event triggered reporting test without gap under non-DRX	1209

A.5.6.7.1.1	Test purpose and Environment	1209

A.5.6.7.1.2	Test Requirements	1212

A.5.6.8	CSI-RS based Inter-frequency Measurements	1212

A.5.6.8.1 	EN-DC event triggered reporting tests for NR FR2 cell when DRX is used	1212

A.5.6.8.1.1	Test Purpose and Environment	1212

A.5.6.8.1.2	Test Requirements	1216

A.5.7	Measurement Performance requirements	1216

A.5.7.1	SS-RSRP	1217

A.5.7.1.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1217

A.5.7.1.1.1	Test Purpose and Environment	1217

A.5.7.1.1.2	Test parameters	1217

A.5.7.1.1.3	Test Requirements	1219

A.5.7.1.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1220

A.5.7.1.2.1	Test Purpose and Environment	1220

A.5.7.1.2.2	Test parameters	1220

A.5.7.1.2.3	Test Requirements	1224

A.5.7.1.3	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	1225

A.5.7.1.3.1	Test Purpose and Environment	1225

A.5.7.1.3.2	Test parameters	1225

A.5.7.1.3.3	Test Requirements	1228

A.5.7.2	SS-RSRQ	1229

A.5.7.2.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1229

A.5.7.2.1.1	Test Purpose and Environment	1229

A.5.7.2.1.2	Test Parameters	1229

A.5.7.2.1.3	Test Requirements	1231

A.5.7.2.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1232

A.5.7.2.2.1	Test Purpose and Environment	1232

A.5.7.2.2.2	Test Parameters	1232

A.5.7.2.2.3	Test Requirements	1234

A.5.7.3	SS-SINR	1234

A.5.7.3.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1234

A.5.7.3.1.1	Test Purpose and Environment	1234

A.5.7.3.1.2	Test Parameters	1234

A.5.7.3.1.3	Test Requirements	1236

A.5.7.3.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1236

A.5.7.3.2.1	Test Purpose and Environment	1236

A.5.7.3.2.2	Test Parameters	1236

A.5.7.3.2.3	Test Requirements	1238

A.5.7.4	L1-RSRP measurement for beam reporting	1238

A.5.7.4.1	SSB based L1-RSRP measurement	1238

A.5.7.4.1.1	Test Purpose and Environment	1238

A.5.7.4.1.2	Test parameters	1239

A.5.7.4.1.3	Test Requirements	1241

A.5.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1242

A.5.7.4.2.1	Test Purpose and Environment	1242

A.5.7.4.2.2	Test parameters	1242

A.5.7.4.2.3	Test Requirements	1244

A.5.7.5	CLI measurements	1245

A.5.7.5.1	EN-DC SRS-RSRP measurement accuracy with FR2 serving cell	1245

A.5.7.5.1.1	Test Purpose and Environment	1245

A.5.7.5.1.2	Test parameters	1245

A.5.7.5.1.3	Test Requirements	1248

A.5.7.5.2	EN-DC CLI-RSSI measurement accuracy with FR2 serving cell	1249

A.5.7.5.2.1	Test Purpose and Environment	1249

A.5.7.5.2.2	Test parameters	1249

A.5.7.5.2.3	Test Requirements	1251

A.5.7.6	L1-SINR measurement for beam reporting	1252

A.5.7.6.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	1252

A.5.7.6.1.1	Test Purpose and Environment	1252

A.5.7.6.1.2	Test parameters	1252

A.5.7.6.1.3	Test Requirements	1254

A.5.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	1254

A.5.7.6.2.1	Test Purpose and Environment	1254

A.5.7.6.2.2	Test parameters	1255

A.5.7.6.2.3	Test Requirements	1257

A.5.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	1258

A.5.7.6.3.1	Test Purpose and Environment	1258

A.5.7.6.3.2	Test parameters	1258

A.5.7.6.3.3	Test Requirements	1260

A.5.7.7	CSI-RSRP	1260

A.5.7.7.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1260

A.5.7.7.1.2	Test parameters	1261

A.5.7.7.1.3	Test Requirements	1263

A.5.7.7.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1264

A.5.7.7.2.1	Test Purpose and Environment	1264

A.5.7.7.2.2	Test parameters	1264

A.5.7.7.2.3	Test Requirements	1268

A.5.7.8	CSI-RSRQ	1269

A.5.7.8.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	1269

A.5.7.8.1.1	Test Purpose and Environment	1269

A.5.7.8.1.2	Test Parameters	1269

A.5.7.8.1.3	Test Requirements	1271

A.5.7.8.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1271

A.5.7.8.2.1	Test Purpose and Environment	1271

A.5.7.8.2.2	Test Parameters	1271

A.5.7.9	CSI-SINR	1273

A.5.7.9.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1273

A.5.7.9.1.1	Test Purpose and Environment	1273

A.5.7.9.1.2	Test Parameters	1273

Table A.5.7.9.1.2-1: CSI-SINR Intra frequency CSI-SINR supported test configurations	1274

A.5.7.9.1.3	Test Requirements	1276

A.5.7.9.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1276

A.5.7.9.2.1	Test Purpose and Environment	1276

A.5.7.9.2.2	Test Parameters	1276

A.5.7.9.2.3	Test Requirements	1279

A.5.8	Void	1279

A.6	NR standalone tests with all NR cells in FR1	1011

A.6.1	SA: RRC_IDLE state mobility	1011

A.6.1.1	Cell re-selection to NR	1011

A.6.1.1.1	Cell reselection to FR1 intra-frequency NR case	1011

A.6.1.1.1.1	Test Purpose and Environment	1011

A.6.1.1.1.2	Test Parameters	1011

A.6.1.1.1.3	Test Requirements	1015

A.6.1.1.2	Cell reselection to FR1 inter-frequency NR case	1015

A.6.1.1.2.1	Test Purpose and Environment	1015

A.6.1.1.2.2	Test Parameters	1015

A.6.1.1.2.3	Test Requirements	1019

A.6.1.1.3	Cell reselection to FR1 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1019

A.6.1.1.3.1	Test Purpose and Environment	1019

A.6.1.1.3.2	Test Parameters	1019

A.6.1.1.3.3	Test Requirements	1024

A.6.1.1.4	Cell reselection to FR1 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1024

A.6.1.1.4.1	Test Purpose and Environment	1024

A.6.1.1.4.2	Test Parameters	1024

A.6.1.1.4.3	Test Requirements	1028

A.6.1.1.5	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1028

A.6.1.1.5.1	Test Purpose and Environment	1028

A.6.1.1.5.2	Test Parameters	1028

A.6.1.1.5.3	Test Requirements	1032

A.6.1.1.6	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1033

A.6.1.1.6.1	Test Purpose and Environment	1033

A.6.1.1.6.2	Test Parameters	1033

A.6.1.1.6.3	Test Requirements	1037

A.6.1.1.7	Cell reselection to FR1 intra-frequency NR case for UE configured with highSpeedMeasFlag-r16	1038

A.6.1.1.7.1	Test Purpose and Environment	1038

A.6.1.1.7.2	Test Parameters	1038

A.6.1.1.7.3	Test Requirements	1042

A.6.1.2	Inter-RAT E-UTRAN cell re-selection	1042

A.6.1.2.1	Cell reselection to higher priority E-UTRAN	1042

A.6.1.2.1.1	Test Purpose and Environment	1042

A.6.1.2.1.2	Test Parameters	1042

A.6.1.2.1.3	Test Requirements	1045

A.6.1.2.2	Cell reselection to lower priority E-UTRAN	1046

A.6.1.2.2.1	Test Purpose and Environment	1046

A.6.1.2.2.2	Test Parameters	1046

A.6.1.2.2.3	Test Requirements	1049

A.6.1.2.3	Cell reselection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion	1050

A.6.1.2.3.1	Test Purpose and Environment	1050

A.6.1.2.3.2	Test Parameters	1050

A.6.1.2.3.3	Test Requirements	1053

A.6.1.2.4	Cell reselection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion	1054

A.6.1.2.4.1	Test Purpose and Environment	1054

A.6.1.2.4.2	Test Parameters	1054

A.6.1.2.4.3	Test Requirements	1057

A.6.1.2.5.3	Test Requirements	1061

A.6.2	SA: RRC_INACTIVE state mobility	1062

A.6.3	RRC_CONNECTED state mobility	1062

A.6.3.1	Handover	1062

A.6.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell	1062

A.6.3.1.1.1	Test Purpose and Environment	1062

A.6.3.1.1.2	Test Parameters	1062

A.6.3.1.1.3 Test Requirements	1066

A.6.3.1.2	Intra-frequency handover from FR1 to FR1; unknown target cell	1066

A.6.3.1.2.1	Test Purpose and Environment	1066

A.6.3.1.2.2	Test Parameters	1066

A.6.3.1.2.3	Test Requirements	1070

A.6.3.1.3	Inter-frequency handover from FR1 to FR1; unknown target cell	1070

A.6.3.1.3.1	Test Purpose and Environment	1070

A.6.3.1.3.2	Test Parameters	1070

A.6.3.1.3.3 Test Requirements	1074

A.6.3.1.4	 SA NR - E-UTRAN handover	1074

A.6.3.1.4.1	Test Purpose and Environment	1074

A.6.3.1.4.2	Test Requirements	1080

A.6.3.1.5	SA NR - E-UTRAN handover with unknown target cell	1080

A.6.3.1.5.1	Test Purpose and Environment	1080

A.6.3.1.5.2	Test Requirements	1086

A.6.3.1.6	 SA NR - UTRAN FDD handover	1086

A.6.3.1.6.1	Test Purpose and Environment	1086

A.6.3.1.6.2	Test Requirements	1090

A.6.3.1.7	Intra-frequency synchronous DAPS handover in FR1	1090

A.6.3.1.7.1	Test Purpose and Environment	1090

A.6.3.1.7.2	Test Parameters	1090

A.6.3.1.7.3 Test Requirements	1094

A.6.3.1.8	Intra-frequency asynchronous DAPS handover in FR1	1095

A.6.3.1.8.1	Test Purpose and Environment	1095

A.6.3.1.8.2	Test Parameters	1095

A.6.3.1.8.3 Test Requirements	1099

A.6.3.1.9	Intra-band inter-frequency synchronous DAPS handover test in SA for FR1	1100

A.6.3.1.9.1	Test Purpose and Environment	1100

A.6.3.1.9.2	Test Parameters	1100

A.6.3.1.9.3 Test Requirements	1104

A.6.3.1.10	Intra-band inter-frequency asynchronous DAPS handover test in SA for FR1	1104

A.6.3.1.10.1	Test Purpose and Environment	1104

A.6.3.1.10.2	Test Parameters	1105

A.6.3.1.10.3 Test Requirements	1108

A.6.3.1.11	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR1	1108

A.6.3.1.11.1	Test Purpose and Environment	1108

A.6.3.1.11.2	Test Parameters	1108

A.6.3.1.11.3 Test Requirements	1115

A.6.3.1.12	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR1	1115

A.6.3.1.12.1	Test Purpose and Environment	1115

A.6.3.1.12.2	Test Parameters	1116

A.6.3.1.12.3 Test Requirements	1122

A.6.3.2	RRC Connection Mobility Control	1122

A.6.3.2.1	SA: RRC Re-establishment	1122

A.6.3.2.1.1	Intra-frequency RRC Re-establishment in FR1	1122

A.6.3.2.1.2	Inter-frequency RRC Re-establishment in FR1	1126

A.6.3.2.1.3	Intra-frequency RRC Re-establishment in FR1 without serving cell timing	1130

A.6.3.2.2	Random Access	1134

A.6.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	1134

A.6.3.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone	1139

A.6.3.2.2.3	2-step RA type contention based random access test in FR1 for NR standalone	1143

A.6.3.2.2.4	2-step RA type non-contention based test in FR1 for NR standalone	1148

A.6.3.2.3	SA: RRC Connection Release with Redirection	1152

A.6.3.2.3.1	Redirection from NR in FR1 to NR in FR1	1152

A.6.3.2.3.2	Redirection from NR in FR1 to E-UTRAN	1156

A.6.3.3	Conditional handover	1163

A.6.3.3.1	Intra-frequency conditional handover from FR1 to FR1	1163

A.6.3.3.1.1	Test Purpose and Environment	1163

A.6.3.3.1.2	Test Parameters	1163

A.6.3.3.1.3 Test Requirements	1167

A.6.3.3.2	Inter-frequency conditional handover from FR1 to FR1	1167

A.6.3.3.2.1	Test Purpose and Environment	1167

A.6.3.3.2.2	Test Parameters	1167

A.6.3.3.2.3 Test Requirements	1171

A.6.4	Timing	1171

A.6.4.1	UE transmit timing	1171

A.6.4.1.1	NR UE Transmit Timing Test for FR1	1171

A.6.4.1.1.1	Test Purpose and environment	1171

A.6.4.1.1.2	Test requirements	1175

A.6.4.2	UE timer accuracy	1175

A.6.4.3	Timing advance	1175

A.6.4.3.1	SA FR1 timing advance adjustment accuracy	1175

A.6.4.3.1.1	Test Purpose and Environment	1175

A.6.4.3.1.2	Test Parameters	1175

A.6.4.3.1.3	Test Requirements	1179

A.6.5	Signalling characteristics	1179

A.6.5.1	Radio link Monitoring	1179

A.6.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	1180

A.6.5.1.1.1	Test Purpose and Environment	1180

A.6.5.1.1.2	Test Requirements	1185

A.6.5.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	1185

A.6.5.1.2.1	Test Purpose and Environment	1185

A.6.5.1.2.2	Test Requirements	1191

A.6.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	1191

A.6.5.1.3.1	Test Purpose and Environment	1191

A.6.5.1.3.2	Test Requirements	1197

A.6.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	1197

A.6.5.1.4.1	Test Purpose and Environment	1197

A.6.5.1.4.2	Test Requirements	1203

A.6.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	1203

A.6.5.1.5.1	Test Purpose and Environment	1203

A.6.5.1.5.2	Test Requirements	1209

A.6.5.1.6	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	1209

A.6.5.1.6.1	Test Purpose and Environment	1209

A.6.5.1.6.2	Test Requirements	1215

A.6.5.1.7	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	1215

A.6.5.1.7.1	Test Purpose and Environment	1215

A.6.5.1.7.2	Test Requirements	1219

A.6.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	1219

A.6.5.1.8.1	Test Purpose and Environment	1219

A.6.5.1.8.2	Test Requirements	1225

A.6.5.2	Interruption	1225

A.6.5.2.1	Interruptions during measurements on deactivated NR SCC in FR1	1225

A.6.5.2.1.2	Test Requirements	1229

A.6.5.2.2	SA interruptions at NR SRS carrier based switching	1230

A.6.5.2.2.1	Test Purpose and Environment	1230

A.6.5.2.2.2	Test Parameters	1230

A.6.5.2.2.3	Test Requirements	1232

A.6.5.3	SCell Activation and Deactivation Delay	1233

A.6.5.3.1	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle	1233

A.6.5.3.1.1	Test Purpose and Environment	1233

A.6.5.3.1.2	Test Requirements	1237

A.6.5.3.2	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle	1238

A.6.5.3.2.1	Test Purpose and Environment	1238

A.6.5.3.2.2	Test Requirements	1238

A.6.5.3.3	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	1238

A.6.5.3.3.1	Test Purpose and Environment	1238

A.6.5.3.3.2	Test Requirements	1239

A.6.5.3.4	Direct SCell activation at SCell addition of known SCell in FR1	1239

A.6.5.3.4.1	 Test Purpose and Environment	1239

A.6.5.3.4.2	 Test Requirements	1243

A.6.5.3.5	Direct SCell activation at handover with known SCell in FR1	1243

A.6.5.3.5.1	Test Purpose and Environment	1243

A.6.5.3.5.2	Test Requirements	1247

A.6.5.4	UE UL carrier RRC reconfiguration Delay	1248

A.6.5.4.1	UE UL carrier RRC reconfiguration Delay	1248

A.6.5.4.1.1	Test Purpose and Environment	1248

A.6.5.4.1.2	Test Requirements	1258

A.6.5.4.2	Void	1259

A.6.5.5	Beam Failure Detection and Link recovery procedures	1259

A.6.5.5.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	1259

A.6.5.5.1.1	Test Purpose and Environment	1259

A.6.5.5.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	1264

A.6.5.5.2.1	Test Purpose and Environment	1264

A.6.5.5.2.2	Test Requirements	1271

A.6.5.5.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	1271

A.6.5.5.3.1	Test Purpose and Environment	1271

A.6.5.5.3.2	Test Requirements	1277

A.6.5.5.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	1277

A.6.5.5.4.1	Test Purpose and Environment	1277

A.6.5.5.4.2	Test Requirements	1283

A.6.5.5.5	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	1283

A.6.5.5.5.1	Test Purpose and Environment	1283

A.6.5.5.5.2	Test Requirements	1287

A.6.5.5.6	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	1288

A.6.5.5.6.1	Test Purpose and Environment	1288

A.6.5.5.6.2	Test Requirements	1292

A.6.5.6	Active BWP switch	1292

A.6.5.6.1	DCI-based and Timer-based Active BWP Switch	1292

A.6.5.6.1.1	NR FR1- NR FR1 DL active BWP switch of SCell with non-DRX in SA	1292

A.6.5.6.1.2	NR FR1 DL active BWP switch with non-DRX in SA	1298

A.6.5.6.2	RRC-based Active BWP Switch	1302

A.6.5.6.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	1302

A.6.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1306

A.6.5.6.3.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	1306

A.6.5.6.3.1.1	Test Purpose and Environment	1306

A.6.5.6.3.1.2	Test Requirements	1311

A.6.5.6.4	SCell dormancy switch	1312

A.6.5.6.4.1	NR FR1 PCell SCell dormancy switch of single FR1 SCell outside active time	1312

A.6.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR1 SCells inside active time	1318

A.6.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1322

A.6.5.6.5.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	1322

A.6.5.7	DL interruptions at switching between two uplink carriers	1326

A.6.5.7.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	1326

A.6.5.7.1.1	Test Purpose and Environment	1326

A.6.5.7.1.2	Test Requirements	1330

A.6.5.7.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	1330

A.6.5.7.2.1	Test Purpose and Environment	1330

A.6.5.7.2.2	Test Requirements	1334

A.6.5.8	UE specific CBW change	1334

A.6.5.8.1	UE specific CBW change on PCell in FR1 in non-DRX	1334

A.6.5.8.1.1	Test Purpose and Environment	1334

A.6.5.8.1.2	Test Requirements	1338

A.6.5.9	Pathloss reference signal switching delay	1338

A.6.5.9.1	MAC-CE based pathloss reference signal switch delay	1338

A.6.5.9.1.1	Test Purpose and Environment	1338

A.6.5.9.1.2	Test Requirements	1341

A.6.6	Measurement procedure	1342

A.6.6.1	Intra-frequency Measurements	1342

A.6.6.1.1	SA event triggered reporting tests without gap under non-DRX	1342

A.6.6.1.1.1	Test purpose and Environment	1342

A.6.6.1.1.2	Test parameters	1342

A.6.6.1.1.3	Test Requirements	1346

A.6.6.1.2	SA event triggered reporting tests without gap under DRX	1346

A.6.6.1.2.1	Test purpose and Environment	1346

A.6.6.1.2.2	Test parameters	1346

A.6.6.1.2.3	Test Requirements	1350

A.6.6.1.3	SA event triggered reporting tests with per-UE gaps under non-DRX	1350

A.6.6.1.3.1	Test purpose and Environment	1350

A.6.6.1.3.2	Test parameters	1350

A.6.6.1.3.3	Test Requirements	1354

A.6.6.1.4	SA event triggered reporting tests with per-UE gaps under DRX	1354

A.6.6.1.4.1	Test purpose and Environment	1354

A.6.6.1.4.2	Test parameters	1354

A.6.6.1.4.3	Test Requirements	1358

A.6.6.1.5	SA event triggered reporting tests without gap under non-DRX with SSB index reading	1358

A.6.6.1.5.1	Test purpose and Environment	1358

A.6.6.1.5.2	Test parameters	1358

A.6.6.1.5.3	Test Requirements	1360

A.6.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading	1361

A.6.6.1.6.1	Test purpose and Environment	1361

A.6.6.1.6.2	Test parameters	1361

A.6.6.1.6.3	Test Requirements	1362

A.6.6.1.7	SA event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	1363

A.6.6.1.7.1	Test purpose and Environment	1363

A.6.6.1.7.2	Test parameters	1363

A.6.6.1.7.3	Test Requirements	1367

A.6.6.2	Inter-frequency Measurements	1367

A.6.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	1367

A.6.6.2.1.1	Test Purpose and Environment	1367

A.6.6.2.1.2	Test Requirements	1371

A.6.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used	1371

A.6.6.2.2.1	Test Purpose and Environment	1371

A.6.6.2.2.2	Test Requirements	1376

A.6.6.2.3	Void	1377

A.6.6.2.4	Void	1377

A.6.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	1377

A.6.6.2.5.1	Test Purpose and Environment	1377

A.6.6.2.5.2	Test Requirements	1381

A.6.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used	1381

A.6.6.2.6.1	Test Purpose and Environment	1381

A.6.6.2.6.2	Test Requirements	1386

A.6.6.2.7	Void	1387

A.6.6.2.8	Void	1387

A.6.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern	1387

A.6.6.2.9.1	Test Purpose and Environment	1387

A.6.6.2.9.2	Test Requirements	1390

A.6.6.2.10	SA event triggered reporting tests for FR1 when DRX is used	1390

A.6.6.2.10.1	Test Purpose and Environment	1390

A.6.6.2.10.2	Test Requirements	1394

A.6.6.2.11	SA event triggered reporting tests for FR1 without gap when DRX is not used	1395

A.6.6.2.11.1	Test Purpose and Environment	1395

A.6.6.2.11.2	Test Requirements	1398

A.6.6.3	Inter-RAT Measurements	1398

A.6.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	1398

A.6.6.3.1.1	Test Purpose and Environment	1398

A.6.6.3.1.2	Test Requirements	1404

A.6.6.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	1404

A.6.6.3.2.1	Test Purpose and Environment	1404

A.6.6.3.2.2	Test Requirements	1411

A.6.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for UE configured with highSpeedMeasFlag-r16	1411

A.6.6.3.3.1	Test Purpose and Environment	1411

A.6.6.3.3.2	Test Requirements	1418

A.6.6.4	L1-RSRP measurement for beam reporting	1418

A.6.6.4.1	SSB based L1-RSRP measurement when DRX is not used	1418

A.6.6.4.1.1	Test Purpose and Environment	1418

A.6.6.4.1.2	Test parameters	1418

A.6.6.4.1.3	Test Requirements	1421

A.6.6.4.2	SSB based L1-RSRP measurement when DRX is used	1421

A.6.6.4.2.1	Test Purpose and Environment	1421

A.6.6.4.2.2	Test parameters	1422

A.6.6.4.2.3	Test Requirements	1425

A.6.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	1425

A.6.6.4.3.1	Test Purpose and Environment	1425

A.6.6.4.3.2	Test parameters	1426

A.6.6.4.3.3	Test Requirements	1429

A.6.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	1429

A.6.6.4.4.1	Test Purpose and Environment	1429

A.6.6.4.4.2	Test parameters	1430

A.6.6.4.4.3	Test Requirements	1433

A.6.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	1433

A.6.6.4.5.1	Test Purpose and Environment	1433

A.6.6.4.5.2	Test parameters	1434

A.6.6.4.5.3	Test Requirements	1437

A.6.6.5	1437

A.6.6.5.1	SA NR - UTRAN FDD event-triggered reporting in non-DRX in FR1	1437

A.6.6.5.1.1	Test Purpose and Environment	1437

A.6.6.5.1.2	Test Requirements	1441

A.6.6.6	CLI measurements	1441

A.6.6.6.1	SRS-RSRP measurement with DRX	1441

A.6.6.6.1.1	Test Purpose and Environment	1441

A.6.6.6.1.2	Test Parameters	1442

A.6.6.6.1.3	Test Requirements	1445

A.6.6.6.2	CLI-RSSI measurement with DRX	1445

A.6.6.6.2.1	Test Purpose and Environment	1445

A.6.6.6.2.2	Test Parameters	1446

A.6.6.6.2.3	Test Requirements	1448

A.6.6.7	NR measurements with autonomous gaps	1448

A.6.6.7.1	SA intra-frequency CGI identification of NR neighbor cell in FR1	1448

A.6.6.7.1.1	Test Purpose and Environment	1448

A.6.6.7.1.2	Test Parameters	1448

A.6.6.7.1.3	Test Requirements	1451

A.6.6.7.2	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA	1451

A.6.6.7.2.1	Test Purpose and Environment	1451

A.6.6.7.2.2	Test Requirements	1454

A.6.6.8	L1-SINR measurement for beam reporting	1455

A.6.6.8.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is used	1455

A.6.6.8.1.1	Test Purpose and Environment	1455

A.6.6.8.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	1458

A.6.6.8.2.1	Test Purpose and Environment	1458

A.6.6.8.2.2	Test parameters	1459

A.6.6.8.2.3	Test Requirements	1463

A.6.6.8.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	1463

A.6.6.8.3.1	Test Purpose and Environment	1463

A.6.6.8.3.2	Test parameters	1464

A.6.6.8.3.3	Test Requirements	1467

A.6.6.9	Idle Mode CA/DC Measurements	1467

A.6.6.9.1	SA Idle mode CA/DC measurement for FR1	1467

A.6.6.9.1.1	Test Purpose and Environment	1467

A.6.6.9.1.2	Test Requirements	1474

A.6.6.10	CSI-RS based intra-frequency Measurements	1474

A.6.6.10.1	SA event triggered reporting tests without gap under non-DRX	1474

A.6.6.10.1.1	Test purpose and Environment	1474

A.6.6.10.1.2	Test Requirements	1477

A.6.6.11	CSI-RS based inter-frequency Measurements	1477

A.6.6.11.1	 SA event triggered reporting tests with gap under DRX	1477

A.6.6.11.1.1	Test Purpose and Environment	1477

A.6.6.11.1.2	Test Requirements	1481

A.6.6.12	RSTD measurements	1481

A. 6.6.12.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA	1481

A. 6.6.12.1.1	Test Purpose and Environment	1481

A.6.6.12.1.2	Test Requirements	1487

A. 6.6.12.2	NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR1 SA	1487

A. 6.6.12.2.1	Test Purpose and Environment	1487

A.6.6.12.2.2	Test Requirements	1493

A.6.6.13 PRS-RSRP measurements	1493

A.6.6.13.1	PRS-RSRP reporting delay test case for single positioning frequency layer	1493

A.6.6.13.1.1	Test purpose and Environment	1493

A.6.6.13.1.2	Test Requirements	1497

A.6.6.13.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	1497

A.6.6.13.2.1	Test purpose and Environment	1497

A.6.6.13.2.2	Test Requirements	1501

A.6.6.14	UE Rx-Tx time difference measurements	1501

A.6.6.14.1 UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	1501

A.6.6.14.1.1	Test purpose and environment	1501

A.6.6.14.1.2	Test requirements	1505

A.6.6.14.2 UE Rx-Tx time difference measurement for dual positioning frequency layers in FR1 SA	1505

A.6.6.14.2.1	Test purpose and environment	1505

A.6.6.14.2.2	Test requirements	1509

A.6.6.15	Idle Mode measurements of inter-RAT CA candidate cells for early reporting	1509

A.6.6.15.1	Test Purpose and Environment	1509

A.6.6.15.2	Test Requirements	1516

A.6.7	Measurement Performance requirements	1517

A.6.7.1	SS-RSRP	1517

A.6.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1517

A.6.7.1.1.1	Test Purpose and Environment	1517

A.6.7.1.1.2	Test parameters	1517

A.6.7.1.1.3	Test Requirements	1522

A.6.7.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1522

A.6.7.1.2.1	Test Purpose and Environment	1522

A.6.7.1.2.2	Test parameters	1522

A.6.7.1.2.3	Test Requirements	1526

A.6.7.1.3	Void	1526

A.6.7.2	SS-RSRQ	1526

A.6.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1526

A.6.7.2.1.1	Test Purpose and Environment	1526

A.6.7.2.1.2	Test Parameters	1527

A.6.7.2.1.3	Test Requirements	1532

A.6.7.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1532

A.6.7.2.2.1	Test Purpose and Environment	1532

A.6.7.2.2.2	Test Parameters	1532

A.6.7.2.2.3	Test Requirements	1537

A.6.7.3	SS-SINR	1537

A.6.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1537

A.6.7.3.1.1	Test Purpose and Environment	1537

A.6.7.3.1.2	Test Parameters	1538

A.6.7.3.1.3	Test Requirements	1542

A.6.7.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1542

A.6.7.3.2.1	Test Purpose and Environment	1542

A.6.7.3.2.2	Test Parameters	1542

A.6.7.3.2.3	Test Requirements	1548

A.6.7.4	L1-RSRP measurement for beam reporting	1549

A.6.7.4.1	SSB based L1-RSRP measurement	1549

A.6.7.4.1.1	Test Purpose and Environment	1549

A.6.7.4.1.2	Test parameters	1549

A.6.7.4.1.3	Test Requirements	1553

A.6.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1553

A.6.7.4.2.1	Test Purpose and Environment	1553

A.6.7.4.2.2	Test parameters	1554

A.6.7.4.2.3	Test Requirements	1558

A.6.7.5	E-UTRAN RSRP	1558

A.6.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell	1558

A.6.7.5.1.1	Test Purpose and Environment	1558

A.6.7.5.1.2	Test parameters	1559

A.6.7.5.1.3	Test Requirements	1565

A.6.7.6	E-UTRAN RSRQ	1566

A.6.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell	1566

A.6.7.6.1.1	Test Purpose and Environment	1566

A.6.7.6.1.2	Test parameters	1566

A.6.7.6.1.3	Test Requirements	1571

A.6.7.7	E-UTRAN RS-SINR	1572

A.6.7.7.1	SA: inter-RAT measurement accuracy with FR1 serving cell	1572

A.6.7.7.1.1	Test Purpose and Environment	1572

A.6.7.7.1.2	Test parameters	1572

A.6.7.7.1.3	Test Requirements	1578

A.6.7.8	CLI measurements	1579

A.6.7.8.1	SA SRS-RSRP measurement accuracy with FR1 serving cell	1579

A.6.7.8.1.1	Test Purpose and Environment	1579

A.6.7.8.1.2	Test parameters	1579

A.6.7.8.1.3	Test Requirements	1585

A.6.7.8.2	SA CLI-RSSI measurement accuracy with FR1 serving cell	1585

A.6.7.8.2.1	Test Purpose and Environment	1585

A.6.7.8.2.2	Test parameters	1586

A.6.7.8.2.3	Test Requirements	1589

A.6.7.9	L1-SINR measurement for beam reporting	1590

A.6.7.9.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	1590

A.6.7.9.1.1	Test Purpose and Environment	1590

A.6.7.9.1.2	Test parameters	1590

A.6.7.9.1.3	Test Requirements	1594

A.6.7.9.2	L1-SINR measurement with SSB based CMR and dedicated IMR	1594

A.6.7.9.2.1	Test Purpose and Environment	1594

A.6.7.9.2.2	Test parameters	1595

A.6.7.9.2.3	Test Requirements	1600

A.6.7.9.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	1600

A.6.7.9.3.1	Test Purpose and Environment	1600

A.6.7.9.3.2	Test parameters	1600

A.6.7.9.3.3	Test Requirements	1605

A.6.7.10	CSI-RSRP	1605

A.6.7.10.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1605

A.6.7.10.1.1	Test Purpose and Environment	1605

A.6.7.10.1.2	Test parameters	1606

A.6.7.10.1.3	Test Requirements	1611

A.6.7.10.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1611

A.6.7.10.2.1	Test Purpose and Environment	1611

A.6.7.10.2.2	Test parameters	1611

A.6.7.10.2.3	Test Requirements	1616

A.6.7.11	CSI-RSRQ	1616

A.6.7.11.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1616

A.6.7.11.1.1	Test Purpose and Environment	1616

A.6.7.11.1.2	Test Parameters	1616

A.6.7.11.1.3	Test Requirements	1621

A.6.7.11.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1621

A.6.7.11.2.1	Test Purpose and Environment	1621

A.6.7.11.2.2	Test Parameters	1621

A.6.7.11.2.3	Test Requirements	1626

A.6.7.12	CSI-SINR	1627

A.6.7.12.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1627

A.6.7.12.1.1	Test Purpose and Environment	1627

A.6.7.12.1.2	Test Parameters	1627

A.6.7.12.1.3	Test Requirements	1631

A.6.7.12.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1632

A.6.7.12.2.1	Test Purpose and Environment	1632

A.6.7.12.2.2	Test Parameters	1632

A.6.7.12.2.3	Test Requirements	1638

A.6.7.13	RSTD measurements	1638

A.6.7.13.1	RSTD measurement accuracy test case for single positioning frequency layer	1638

A.6.7.13.1.1	Test purpose and Environment	1638

A.6.7.13.1.2	Test Requirements	1641

A.6.7.13.2	RSTD measurement accuracy test case for dual positioning frequency layer	1641

A.6.7.13.2.1	Test purpose and Environment	1641

A.6.7.13.2.2	Test Requirements	1645

A.6.7.14	PRS-RSRP measurements	1645

A.6.7.14.1	SA: measurement accuracy with PRS in FR1	1645

A.6.7.14.1.1	Test Purpose and Environment	1645

A.6.7.14.1.2	Test parameters	1645

A.6.7.14.1.3	Test Requirements	1651

A.6.7.15	UE Rx-Tx time difference measurements	1651

A.6.7.15.1 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA	1651

A.6.7.15.1.1	Test purpose and environment	1651

A.6.7.15.1.2	Test parameters	1652

A.6.7.15.1.3	Test requirements	1655

A.7.1	SA: RRC_IDLE state mobility	1655

A.7.1.1	Cell re-selection to NR	1655

A.7.1.1.1	Cell reselection to FR2 intra-frequency NR case	1655

A.7.1.1.1.1	Test Purpose and Environment	1655

A.7.1.1.1.2	Test Parameters	1655

A.7.1.1.1.3	Test Requirements	1659

A.7.1.1.2	Cell reselection to FR2 inter-frequency NR case	1659

A.7.1.1.2.1	Test Purpose and Environment	1659

A.7.1.1.2.2	Test Parameters	1659

A.7.1.1.2.3	Test Requirements	1663

A.7.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1663

A.7.1.1.3.1	Test Purpose and Environment	1663

A.7.1.1.3.2	Test Parameters	1663

A.7.1.1.3.3	Test Requirements	1666

A.7.1.1.4	Cell reselection to FR2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1666

A.7.1.1.4.1	Test Purpose and Environment	1666

A.7.1.1.4.2	Test Parameters	1666

A.7.1.1.4.3	Test Requirements	1669

A.7.1.1.5	Cell reselection to FR2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1669

A.7.1.1.5.1	Test Purpose and Environment	1669

A.7.1.1.5.2	Test Parameters	1669

A.7.1.1.5.3	Test Requirements	1673

A.7.1.1.6	Cell reselection to FR2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1673

A.7.1.1.6.1	Test Purpose and Environment	1673

A.7.1.1.6.2	Test Parameters	1673

A.7.1.1.6.3	Test Requirements	1675

A.7.2	SA: RRC_INACTIVE state mobility	1676

A.7.3	RRC_CONNECTED state mobility	1676

A.7.3.1	Handover	1676

A.7.3.1.1	Inter-frequency handover from FR1 to FR2; unknown target cell	1676

A.7.3.1.1.1	Test Purpose and Environment	1676

A.7.3.1.1.2	Test Parameters	1676

A.7.3.1.1.3	Test Requirements	1680

A.7.3.1.2	Intra-frequency handover from FR2 to FR2; unknown target cell	1680

A.7.3.1.2.1	Test Purpose and Environment	1680

A.7.3.1.2.2	Test Parameters	1680

A.7.3.1.2.3	Test Requirements	1683

A.7.3.1.3	Inter-frequency handover from FR2 to FR2; unknown target cell	1683

A.7.3.1.3.1	Test Purpose and Environment	1683

A.7.3.1.3.2	Test Parameters	1683

A.7.3.1.3.3 Test Requirements	1684

A.7.3.1.4	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR2	1685

A.7.3.1.4.1	Test Purpose and Environment	1685

A.7.3.1.4.2	Test Parameters	1685

A.7.3.1.4.3 Test Requirements	1692

A.7.3.1.5	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR2	1692

A.7.3.1.5.1	Test Purpose and Environment	1692

A.7.3.1.5.2	Test Parameters	1692

A.7.3.1.5.3 Test Requirements	1699

A.7.3.2	RRC Connection Mobility Control	1699

A.7.3.2.1	SA: RRC Re-establishment	1699

A.7.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	1699

A.7.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	1702

A.7.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	1705

A.7.3.2.1.3.1	Test Purpose and Environment	1705

A.7.3.2.1.3.2	Test Requirements	1707

A.7.3.2.2	Random Access	1708

A.7.3.2.2.1	4-step RA type contention based random access test in FR2 for NR Standalone	1708

A.7.3.2.2.2	4-step RA type non-contention based random access test in FR2 for NR Standalone	1712

A.7.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	1717

A.7.3.2.2.4	2-step RA type non-contention based random access test in FR2 for NR Standalone	1720

A.7.3.2.3	SA: RRC Connection Release with Redirection	1723

A.7.3.2.3.1	Redirection from NR in FR2 to NR in FR2	1723

A.7.3.3	Conditional Handover	1727

A.7.3.3.1	Intra-frequency conditional handover from FR2 to FR2	1727

A.7.3.3.1.1	Test Purpose and Environment	1727

A.7.3.3.1.2	Test Parameters	1727

A.7.3.3.1.2.3	Test Requirements	1730

A.7.3.3.2	Inter-frequency conditional handover from FR2 to FR2; unknown target cell	1730

A.7.3.3.2.1	Test Purpose and Environment	1730

A.7.3.3.2.2	Test Parameters	1730

A.7.3.3.2.3 Test Requirements	1733

A.7.4	Timing	1733

A.7.4.1	UE transmit timing	1733

A.7.4.1.1	NR UE Transmit Timing Test for FR2	1733

A.7.4.1.1.1	Test Purpose and environment	1733

A.7.4.1.1.2	Test requirements	1736

A.7.4.2	UE timer accuracy	1736

A.7.4.3	Timing advance	1736

A.7.4.3.1	SA FR2 timing advance adjustment accuracy	1736

A.7.4.3.1.1	Test Purpose and Environment	1736

A.7.4.3.1.2	Test Parameters	1736

A.7.4.3.1.3 Test Requirements	1739

A.7.5	Signaling characteristics	1740

A.7.5.1	Radio link Monitoring	1740

A.7.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	1740

A.7.5.1.1.1	Test Purpose and Environment	1740

A.7.5.1.1.2	Test Requirements	1743

A.7.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	1744

A.7.5.1.2.1	Test Purpose and Environment	1744

A.7.5.1.2.2	Test Requirements	1748

A.7.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	1749

A.7.5.1.3.1	Test Purpose and Environment	1749

A.7.5.1.3.2	Test Requirements	1753

A.7.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	1753

A.7.5.1.4.1	Test Purpose and Environment	1753

A.7.5.1.4.2	Test Requirements	1758

A.7.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	1758

A.7.5.1.5.1	Test Purpose and Environment	1758

A.7.5.1.5.2	Test Requirements	1763

A.7.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	1763

A.7.5.1.6.1	Test Purpose and Environment	1763

A.7.5.1.6.2	Test Requirements	1767

A.7.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	1767

A.7.5.1.7.1	Test Purpose and Environment	1767

A.7.5.1.7.2	Test Requirements	1771

A.7.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	1771

A.7.5.1.8.1	Test Purpose and Environment	1771

A.7.5.1.8.2	Test Requirements	1776

A.7.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	1776

A.7.5.1.9.1	Test Purpose and Environment	1776

A.7.5.1.9.2	Test Requirements	1779

A.7.5.2	Interruption	1779

A.7.5.2.1	Interruptions during measurements on deactivated NR SCC in FR2	1779

A.7.5.2.2	SA interruptions at NR SRS carrier-based switching	1783

A.7.5.2.2.1	Test Purpose and Environment	1783

A.7.5.2.2.2	Test Parameters	1783

A.7.5.2.2.3	Test Requirements	1785

A.7.5.3	SCell Activation and Deactivation Delay	1785

A.7.5.3.1	SCell Activation and deactivation for SCell in FR2 intra-band in non-DRX	1785

A.7.5.3.1.1	Test Purpose and Environment	1785

A.7.5.3.1.2	Test Requirements	1787

A.7.5.3.2	SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2	1787

A.7.5.3.2.1	Test Purpose and Environment	1787

A.7.5.3.2.2	Test Requirements	1791

A.7.5.3.3	SCell Activation and deactivation for SCell in FR2 inter-band in non-DRX	1792

A.7.5.3.3.1	Test Purpose and Environment	1792

A.7.5.3.3.2	Test Requirements	1795

A.7.5.3.4	Direct SCell activation at SCell addition of known SCell in FR2	1796

A.7.5.3.4.1	Test Purpose and Environment	1796

A.7.5.3.4.2	Test Requirements	1799

A.7.5.3.5	Direct SCell activation at handover with known SCell in FR2	1800

A.7.5.3.5.1	Test Purpose and Environment	1800

A.7.5.3.5.2	Test Requirements	1802

A.7.5.4	Void	1803

A.7.5.5	Beam Failure Detection and Link recovery procedures	1803

A.7.5.5.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	1803

A.7.5.5.1.1	Test Purpose and Environment	1803

A.7.5.5.1.2	Test Requirements	1807

A.7.5.5.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	1807

A.7.5.5.2.1	Test Purpose and Environment	1807

A.7.5.5.2.2	Test Requirements	1811

A.7.5.5.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	1811

A.7.5.5.3.1	Test Purpose and Environment	1811

A.7.5.5.3.2	Test Requirements	1815

A.7.5.5.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	1815

A.7.5.5.4.1	Test Purpose and Environment	1815

A.7.5.5.4.2	Test Requirements	1819

A.7.5.5.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	1819

A.7.5.5.5.1	Test Purpose and Environment	1819

A.7.5.5.5.2	Test Requirements	1823

A.7.5.5.6	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	1823

A.7.5.5.6.1	Test Purpose and Environment	1823

A.7.5.5.6.2	Test Requirements	1827

A.7.5.5.7	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	1827

A.7.5.5.7.1	Test Purpose and Environment	1827

A.7.5.5.7.2	Test Requirements	1831

A.7.5.6	Active BWP switch	1831

A.7.5.6.1	DCI-based and Timer-based Active BWP Switch	1831

A.7.5.6.1.1	NR FR2- NR FR2 DL active BWP switch of SCell with non-DRX in SA	1831

A.7.5.6.1.2	NR FR1- NR FR2 DL active BWP switch of SCell with non-DRX in SA A.7.5.6.1.2.1 Test Purpose and Environment	1836

A.7.5.6.1.3	NR FR2 DL active BWP switch with non-DRX in SA	1841

A.7.5.6.1.3.1	Test Purpose and Environment	1841

A.7.5.6.1.3.2	Test Requirements	1844

A.7.5.6.2	RRC-based Active BWP Switch	1844

A.7.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1848

A.7.5.6.3.1	Active BWP switch on multiple SCells with non-DRX in SA	1848

A.7.5.6.4	SCell dormancy switch	1851

A.7.5.6.4.1	NR FR2 PCell SCell dormancy switch of single FR2 SCell inside active time	1851

A.7.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR2 SCells outside active time	1856

A.7.5.6.4.2.2	 Test Requirements	1861

A.7.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1861

A.7.5.6.5.1	Active BWP switch on multiple SCells with non-DRX in SA	1861

A.7.5.7	PSCell addition and release delay	1866

A.7.5.7.1	Addition and Release Delay of known NR PSCell	1866

A.7.5.7.1.1	Test Purpose and Environment	1866

A.7.5.7.2	Addition and Release Delay of unknown NR PSCell	1869

A.7.5.7.2.1	Test Purpose and Environment	1869

A.7.5.7.2.2	Test Requirements	1872

A.7.5.8	Active TCI state switch delay	1872

A.7.5.8.1	MAC-CE based active TCI state switch	1872

A.7.5.8.2	RRC based active TCI state switch	1876

A.7.5.9	Uplink spatial relation switch delay	1880

A.7.5.9.1	MAC-CE based Spatial Relation switch	1880

A.7.5.9.1.1	 NR PCell FR2 spatial relation associated with known DL-RS	1880

A.7.5.9.1.1.2	Test Requirements	1883

A.7.5.9.2	RRC based spatial relation switch	1883

A.7.5.9.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	1883

A.7.5.9.2.1.1	Test Purpose and Environment	1883

A.7.5.9.2.1.2	Test Requirements	1886

A.7.5.10	UE specific CBW change	1886

A.7.5.10.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	1886

A.7.5.10.1.1	Test Purpose and Environment	1886

A.7.5.10.1.2	Test Requirements	1889

A.7.6	Measurement procedure	1890

A.7.6.1	Intra-frequency Measurements	1890

A.7.6.1.1	SA event triggered reporting test without gap under non-DRX	1890

A.7.6.1.1.1	Test purpose and Environment	1890

A.7.6.1.1.2	Test Requirements	1892

A.7.6.1.2	SA event triggered reporting test without gap under DRX	1893

A.7.6.1.2.1	Test purpose and Environment	1893

A.7.6.1.2.2	Test Requirements	1895

A.7.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	1896

A.7.6.1.3.1	Test purpose and Environment	1896

A.7.6.1.3.2	Test Requirements	1899

A.7.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	1899

A.7.6.1.4.1	Test purpose and Environment	1899

A.7.6.1.4.2	Test Requirements	1902

A.7.6.2	Inter-frequency Measurements	1903

A.7.6.2.1	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2)	1903

A.7.6.2.1.1	Test Purpose and Environment	1903

A.7.6.2.1.2	Test Requirements	1906

A.7.6.2.2	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is used (PCell in FR2)	1906

A.7.6.2.2.1	Test Purpose and Environment	1906

A.7.6.2.2.2	Test Requirements	1910

A.7.6.2.3	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is not used (PCell in FR2)	1910

A.7.6.2.3.1	Test Purpose and Environment	1910

A.7.6.2.3.2	Test Requirements	1914

A.7.6.2.4	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is used (PCell in FR2)	1914

A.7.6.2.4.1	Test Purpose and Environment	1914

A.7.6.2.4.2	Test Requirements	1918

A.7.6.2.5	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1)	1918

A.7.6.2.5.1	Test Purpose and Environment	1918

A.7.6.2.5.2	Test Requirements	1922

A.7.6.2.6	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (PCell in FR1)	1923

A.7.6.2.6.1	Test Purpose and Environment	1923

A.7.6.2.6.2	Test Requirements	1927

A.7.6.2.7	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1)	1928

A.7.6.2.7.1	Test Purpose and Environment	1928

A.7.6.2.7.2	Test Requirements	1932

A.7.6.2.8	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1)	1933

A.7.6.2.8.1	Test Purpose and Environment	1933

A.7.6.2.8.2	Test Requirements	1937

A.7.6.2.9	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2) (rel16 additional mandatory gap pattern 17)	1938

A.7.6.2.9.1	Test Purpose and Environment	1938

A.7.6.2.9.2	Test Requirements	1941

A.7.6.2.10	SA event triggered reporting test without gap under non-DRX	1941

A.7.6.2.10.1	Test Purpose and Environment	1941

A.7.6.2.10.2	Test Requirements	1943

A.7.6.2.11	SA event triggered reporting test without gap under DRX	1943

A.7.6.2.11.1	Test Purpose and Environment	1943

A.7.6.2.11.2	Test Requirements	1946

A.7.6.3	L1-RSRP measurement for beam reporting	1947

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1947

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1947

A.7.6.3.1.1	Test Purpose and Environment	1947

A.7.6.3.1.2	Test parameters	1947

A.7.6.3.1.3	Test Requirements	1949

A.7.6.3.2	SSB based L1-RSRP measurement when DRX is used	1949

A.7.6.3.2.1	Test Purpose and Environment	1949

A.7.6.3.2.2	Test parameters	1950

A.7.6.3.2.3	Test Requirements	1952

A.7.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	1952

A.7.6.3.3.1	Test Purpose and Environment	1952

A.7.6.3.3.2	Test parameters	1953

A.7.6.3.3.3	Test Requirements	1955

A.7.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	1956

A.7.6.3.4.1	Test Purpose and Environment	1956

A.7.6.3.4.2	Test parameters	1956

A.7.6.3.3.3	Test Requirements	1958

A.7.6.4	CLI measurements	1959

A.7.6.4.1	SRS-RSRP measurement with non-DRX	1959

A.7.6.4.1.1	Test Purpose and Environment	1959

A.7.6.4.1.2	Test Parameters	1959

A.7.6.4.1.3	Test Requirements	1961

A.7.6.4.2	CLI-RSSI measurement with non-DRX	1961

A.7.6.4.2.1	Test Purpose and Environment	1961

A.7.6.4.2.2	Test Parameters	1962

A.7.6.4.2.3	Test Requirements	1963

A.7.6.5	NR Measurements with autonomous gaps	1964

A.7.6.5.1	SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2)	1964

A.7.6.5.1.1	Test Purpose and Environment	1964

A.7.6.5.1.2	Test Requirements	1967

A.7.6.6	L1-SINR measurement for beam reporting	1967

A.7.6.6.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is not used	1967

A.7.6.6.1.1	Test Purpose and Environment	1967

A.7.6.6.1.2	Test parameters	1967

A.7.6.6.1.3	Test Requirements	1969

A.7.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	1969

A.7.6.6.2.1	Test Purpose and Environment	1969

A.7.6.6.2.2	Test parameters	1970

A.7.6.6.2.3	Test Requirements	1972

A.7.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	1973

A.7.6.6.3.1	Test Purpose and Environment	1973

A.7.6.6.3.2	Test parameters	1973

A.7.6.6.3.3	Test Requirements	1975

A.7.6.7	CSI-RS based intra-frequency Measurements	1975

A.7.6.7.1	SA event triggered reporting test without gap under DRX for CSI-RS based intra-frequency measurement	1975

A.7.6.7.1.1	Test purpose and Environment	1975

A.7.6.7.1.2	Test Requirements	1978

A.7.6.8	CSI-RS based inter-frequency Measurements	1978

A.7.6.8.1	SA event triggered reporting tests for FR2 CSI-RS based measurement when non-DRX is used (PCell in FR2)	1978

A.7.6.8.1.1	Test Purpose and Environment	1978

A.7.6.2.2.2	Test Requirements	1982

A.7.6.9	RSTD measurements	1982

A.7.6.9.1	 NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA	1982

A.7.6.9.1.1	Test Purpose and Environment	1982

A.7.6.9.1.2	Test Requirements	1986

A.7.6.9.2	 NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR2 SA	1987

A.7.6.9.2.1	Test Purpose and Environment	1987

A.7.6.9.2.2	Test Requirements	1992

A.7.6.10 PRS-RSRP measurements	1993

A.7.6.10.1 PRS-RSRP reporting delay test case for single positioning frequency layer	1993

A.7.6.10.1.1	Test Purpose and Environment	1993

A.7.6.10.1.2	Test Requirements	1997

A.7.6.10.2 PRS-RSRP reporting delay test case for dual positioning frequency layer	1997

A.7.6.10.2.1	Test Purpose and Environment	1997

A.7.6.10.2.2	Test Requirements	2001

A.7.6.11	UE Rx-Tx time difference measurements	2001

A.7.6.11.1 UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	2001

A.7.6.11.1.1	Test purpose and environment	2001

A.7.6.11.1.2	Test requirements	2003

A.7.6.11.2 UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR2 SA	2004

A.7.6.11.2.1	Test purpose and environment	2004

A.7.6.11.2.2	Test requirements	2006

A.7.7	Measurement Performance requirements	2007

A.7.7.1	SS-RSRP	2007

A.7.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2007

A.7.7.1.1.1	Test Purpose and Environment	2007

A.7.7.1.1.2	Test parameters	2007

A.7.7.1.1.3	Test Requirements	2011

A.7.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2012

A.7.7.1.2.1	Test Purpose and Environment	2012

A.7.7.1.2.2	Test parameters	2012

A.7.7.1.2.3	Test Requirements	2016

A.7.7.1.3	SA inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	2017

A.7.7.1.3.1	Test Purpose and Environment	2017

A.7.7.1.3.2	Test parameters	2017

A.7.7.1.3.3	Test Requirements	2021

A.7.7.2	SS-RSRQ	2021

A.7.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	2021

A.7.7.2.1.1	Test Purpose and Environment	2021

A.7.7.2.1.2	Test Parameters	2021

A.7.7.2.1.3	Test Requirements	2023

A.7.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2023

A.7.7.2.2.1	Test Purpose and Environment	2023

A.7.7.2.2.2	Test Parameters	2023

A.7.7.2.2.3	Test Requirements	2025

A.7.7.3	SS-SINR	2025

A.7.7.3.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2025

A.7.7.3.1.1	Test Purpose and Environment	2025

A.7.7.3.1.2	Test Parameters	2025

A.7.7.3.1.3	Test Requirements	2028

A.7.7.3.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2028

A.7.7.3.2.1	Test Purpose and Environment	2028

A.7.7.3.2.2	Test Parameters	2028

A.7.7.3.2.3	Test Requirements	2030

A.7.7.4	L1-RSRP measurement for beam reporting	2030

A.7.7.4.1	SSB based L1-RSRP measurement	2030

A.7.7.4.1.1	Test Purpose and Environment	2030

A.7.7.4.1.2	Test parameters	2031

A.7.7.4.1.3	Test Requirements	2033

A.7.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	2034

A.7.7.4.2.1	Test Purpose and Environment	2034

A.7.7.4.2.2	Test parameters	2034

A.7.7.4.2.3	Test Requirements	2036

A.7.7.5	CLI measurements	2037

A.7.7.5.1	SA SRS-RSRP measurement accuracy with FR2 serving cell	2037

A.7.7.5.1.1	Test Purpose and Environment	2037

A.7.7.5.1.2	Test parameters	2037

A.7.7.5.1.3	Test Requirements	2040

A.7.7.5.2	SA CLI-RSSI measurement accuracy with FR2 serving cell	2041

A.7.7.5.2.1	Test Purpose and Environment	2041

A.7.7.5.2.2	Test parameters	2041

A.7.7.5.2.3	Test Requirements	2043

A.7.7.6	L1-SINR measurement for beam reporting	2044

A.7.7.6.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	2044

A.7.7.6.1.1	Test Purpose and Environment	2044

A.7.7.6.1.2	Test parameters	2044

A.7.7.6.1.3	Test Requirements	2046

A.7.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	2046

A.7.7.6.2.1	Test Purpose and Environment	2046

A.7.7.6.2.2	Test parameters	2047

A.7.7.6.2.3	Test Requirements	2049

A.7.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	2049

A.7.7.6.3.1	Test Purpose and Environment	2049

A.7.7.6.3.2	Test parameters	2050

A.7.7.6.3.3	Test Requirements	2052

A.7.7.7	CSI-RSRP	2052

A.7.7.7.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2052

A.7.7.7.1.1	Test Purpose and Environment	2052

A.7.7.7.1.2	Test parameters	2053

A.7.7.7.1.3	Test Requirements	2056

A.7.7.7.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2057

A.7.7.7.2.1	Test Purpose and Environment	2057

A.7.7.7.2.2	Test parameters	2057

A.7.7.7.2.3	Test Requirements	2060

A.7.7.8	CSI-RSRQ	2060

A.7.7.8.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	2060

A.7.7.8.1.1	Test Purpose and Environment	2060

A.7.7.8.1.2	Test Parameters	2061

A.7.7.8.1.3	Test Requirements	2063

A.7.7.8.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2063

A.7.7.8.2.1	Test Purpose and Environment	2063

A.7.7.8.2.2	Test Parameters	2063

A.7.7.8.2.3	Test Requirements	2065

A.7.7.9	CSI-SINR	2065

A.7.7.9.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2065

A.7.7.9.1.1	Test Purpose and Environment	2065

A.7.7.9.1.2	Test Parameters	2065

A.7.7.9.1.3	Test Requirements	2068

A.7.7.9.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2068

A.7.7.9.2.2	Test Parameters	2068

A.7.7.9.2.3	Test Requirements	2070

A.7.7.10	RSTD measurements	2070

A.7.7.10.1	RSTD measurement accuracy test case for single positioning frequency layer	2070

A.7.7.10.1.1	Test purpose and Environment	2070

A.7.7.10.1.2	Test Requirements	2073

A.7.7.10.2	RSTD measurement accuracy test case for dual positioning frequency layer	2073

A.7.7.10.2.1	Test purpose and Environment	2073

A.7.7.10.2.2	Test Requirements	2075

A.7.7.11	PRS-RSRP measurements	2075

A.7.7.11.1	SA measurement accuracy with PRS in FR2	2075

A.7.7.11.1.1	Test Purpose and Environment	2075

A.7.7.11.1.2	Test parameters	2075

A.7.7.11.1.3	Test Requirements	2078

A.7.7.12	UE Rx-Tx time difference measurements	2079

A.7.7.12.1 UE Rx-Tx time difference measurement period for single positioning frequency layer in FR2 SA	2079

A.7.7.12.1.1	Test purpose and environment	2079

A.7.7.12.1.2	Test parameters	2079

A.7.7.12.1.3	Test requirements	2082

A.8	E-UTRA standalone tests for NR RRM	2082

A.8.1	Void	2082

A.8.2	RRC_IDLE state mobility	2082

A.8.2.1	Inter-RAT NR Cell re-selection	2082

A.8.2.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	2082

A.8.2.1.1.1	Test Purpose and Environment	2082

A.8.2.1.1.2	Test Requirements	2087

A.8.2.1.2	E-UTRA Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16	2088

A.8.2.1.2.1	Test Purpose and Environment	2088

A.8.2.1.2.2	Test Requirements	2093

A.8.2.2	E-UTRA – NR Inter-RAT Early Measruement Reporting	2094

A.8.2.2.1	E-UTRA – NR Early Measruement Reporting for NR in FR1	2094

A.8.2.2.1.1	Test Purpose and Environment	2094

A.8.2.2.1.2	Test Requirements	2099

A.8.2.2.2	E-UTRA – NR Early Measruement Reporting for NR in FR2	2099

A.8.2.2.2.1	Test Purpose and Environment	2099

A.8.2.2.2.2	Test Requirements	2101

A.8.3	RRC_CONNECTED state mobility	2102

A.8.3.1	Handover	2102

A.8.3.1.1	E-UTRAN - NR handover in FR1	2102

A.8.3.1.1.1	Test Purpose and Environment	2102

A.8.3.1.1.2	Test Requirements	2107

A.8.4	Measurement procedure	2107

A.8.4.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay	2107

A.8.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay in non-DRX	2107

A.8.4.1.1.1	Test Purpose and Environment	2107

A.8.4.1.1.2	Test Requirements	2109

A.8.4.1.2	E-UTRA – NR Inter-RAT SFTD Measurement Delay in DRX	2110

A.8.4.1.2.1	Test Purpose and Environment	2110

A.8.4.1.2.2	Test Requirements	2111

A.8.4.2	E-UTRA – NR Inter-RAT Measurements	2111

A.8.4.2.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2111

A.8.4.2.1.1	Test Purpose and Environment	2111

A.8.4.2.1.2	Test Requirements	2116

A.8.4.2.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2116

A.8.4.2.2.1	Test Purpose and Environment	2116

A.8.4.2.2.2	Test Requirements	2120

A.8.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2120

A.8.4.2.3.1	Test Purpose and Environment	2120

A.8.4.2.3.2	Test Requirements	2125

A.8.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2125

A.8.4.2.4.1	Test Purpose and Environment	2125

A.8.4.2.4.2	Test Requirements	2129

A.8.4.2.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	2129

A.8.4.2.5.1	Test Purpose and Environment	2129

A.8.4.2.5.2	Test Requirements	2131

A.8.4.2.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	2132

A.8.4.2.6.1	Test Purpose and Environment	2132

A.8.4.2.6.2	Test Requirements	2134

A.8.4.2.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	2135

A.8.4.2.7.1	Test Purpose and Environment	2135

A.8.4.2.7.2	Test Requirements	2137

A.8.4.2.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	2138

A.8.4.2.8.1	Test Purpose and Environment	2138

A.8.4.2.8.2	Test Requirements	2140

A.8.4.2.9	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16	2141

A.8.4.2.9.1	Test Purpose and Environment	2141

A.8.4.2.9.2	Test Requirements	2147

A.8.5	Measurement performance	2147

A.8.5.1	SFTD accuracy	2147

A.8.5.1.1	SFTD accuracy	2147

A.8.5.1.1.1	Test Purpose	2147

A.8.5.1.1.2	Test Environment	2147

A.8.5.1.1.3	Test Requirements	2153

A.8.5.2	E-UTRA – NR Inter-RAT Measurement Performance requirements	2153

A.8.5.2.1.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	2153

A.8.5.2.1.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	2158

A.8.5.2.1.2.1	Test Purpose and Environment	2158

A.8.5.2.1.2.2	Test Parameters	2158

A.8.5.2.1.2.3	Test Requirements	2160

A.8.5.2.2	SS-RSRQ	2160

A.8.5.2.2.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	2160

A.8.5.2.2.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	2165

A.8.5.2.2.2.1	Test Purpose and Environment	2165

A.8.5.2.2.2.2	Test Parameters	2165

A.8.5.2.2.2.3	Test Requirements	2167

A.8.5.2.3	SS-SINR	2167

A.8.5.2.3.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	2167

A.8.5.2.3.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	2171

A.8.5.2.3.2.1	Test Purpose and Environment	2171

A.8.5.2.3.2.2	Test Parameters	2171

A.8.5.2.3.2.3	Test Requirements	2173

A.9	V2X Tests	2174

A.9.1	V2X Tests in FR1	2174

A.9.1.1	Test for V2X UE Transmit Timing	2174

A.9.1.1.1	Test for GNSS as Synchronization Reference Source	2174

A.9.1.1.1.1	Test Purpose and Environment	2174

A.9.1.1.1.2	Test requirements	2174

A.9.1.1.2	Test for SyncRef UE as Synchronization Reference Source	2174

A.9.1.1.2.1	Test Purpose and Environment	2174

A.9.1.1.2.2	Test requirements	2175

A.9.1.1.3	Test for FR1 NR Cell as Synchronization Reference Source	2175

A.9.1.1.3.1	Test Purpose and Environment	2175

A.9.1.1.3.2	Test requirements	2178

A.9.1.2	Test for Initiation/Cease of S-SSB Transmission with V2X Sidelink Communication	2178

A.9.1.2.1	Test for FR1 NR Cell as synchronization reference source without gap under non-DRX	2178

A.9.1.2.1.1	Test Purpose and Environment	2178

A.9.1.2.1.2	Test Requirements	2182

A.9.1.2.2	Test for SyncRef UE as synchronization reference source	2182

A.9.1.2.2.1	Test Purpose and Environment	2182

A.9.1.2.2.2	Test Requirements	2183

A.9.1.3	Test for V2X Synchronization Reference Selection/Reselection	2184

A.9.1.3.1	Test for GNSS configured as the highest priority	2184

A.9.1.3.1.1	Test Purpose and Environment	2184

A.9.1.3.1.2	Test Requirements	2186

A.9.1.3.2	 Test for FR1 NR Cell configured as the highest priority	2187

A.9.1.3.2.1	Test Purpose and Environment	2187

A.9.1.3.2.2	Test Requirements	2189

A.9.1.4	Test for L1 SL-RSRP Measurement	2190

A.9.1.4.1	Test for V2X UE Autonomous Resource Selection/Reselection	2190

A.9.1.4.1.1	Test Purpose and Environment	2190

A.9.1.4.1.2	Test Requirements	2193

A.9.1.4.2	Test for V2X UE Resource Pre-emption	2193

A.9.1.4.2.1	Test Purpose and Environment	2193

A.9.1.4.2.2	Test Requirements	2197

A.9.1.4.3	Test for V2X UE Resource Re-evaluation	2197

A.9.1.4.3.1	Test Purpose and Environment	2197

A.9.1.4.3.2	Test Requirements	2202

A.9.1.5	Test for Congestion Control Measurement	2202

A.9.1.5.1	Test Purpose and Environment	2202

A.9.1.5.2	Test Requirements	2208

A.9.1.6	Test for Interruption	2208

A.9.1.6.1	Test for Interruption to WAN due to V2X Sidelink Communication	2208

A.9.1.6.1.1	Test Purpose and Environment	2208

A.9.1.6.1.2	Test Requirements	2211

A.10	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	2211

A.10.1	RRC_CONNECTED state mobility	2211

A.10.1.1	RRC connection mobility control	2211

A.10.1.1.1	Random Access	2211

A.10.1.1.1.1	4-step RA type contention-based random access for NR PSCell with CCA	2211

A.10.1.1.1.1.1	Test Purpose and Environment	2211

A.10.1.1.1.1.2	Test Requirements	2214

A.10.1.1.1.1.2.1	Random Access Preamble Transmission	2214

A.10.1.1.1.1.2.2	Random Access Response Reception	2214

A.10.1.1.1.1.2.3	No Random Access Response Reception	2215

A.10.1.1.1.1.2.4	Receiving an UL grant for msg3 retransmission	2215

A.10.1.1.1.1.2.5	 Contention Resolution Timer expiry	2215

A.10.1.1.1.2	4-step RA type non-contention based random access for NR PSCell with CCA	2216

A.10.1.1.1.2.1	Test Purpose and Environment	2216

A.10.1.1.1.2.2	Test Requirements	2219

A.10.1.1.1.2.2.1	SSB-based Random Access Preamble Transmission	2219

A.10.1.1.1.2.2.2	Random Access Response Reception	2220

A.10.1.1.1.2.2.3	No Random Access Response Reception	2220

A.10.1.1.1.3	2-step RA type contention-based random access for NR PSCell with CCA	2220

A.10.1.1.1.3.1	Test Purpose and Environment	2220

A.10.1.1.1.3.2	Test Requirements	2224

A.10.1.1.1.3.2.1	MsgA Transmission	2224

A.10.1.1.1.3.2.2	MsgB Reception	2224

A.10.1.1.1.3.2.3	No MsgB Reception	2225

A.10.1.1.1.4	2-step RA type non-contention based random access for NR PSCell with CCA	2225

A.10.1.1.1.4.1	Test Purpose and Environment	2225

A.10.1.1.1.4.2	Test Requirements	2229

A.10.1.1.1.4.2.1	MsgA Transmission	2229

A.10.1.1.1.4.2.2	MsgB Reception	2230

A.10.1.1.1.4.2.3	No MsgB Reception	2230

A.10.2	Timing	2231

A.10.2.1	UE transmit timing	2231

A.10.2.2	UE timing advance	2235

A.10.3	Signalling characteristics	2238

A.10.3.1	Radio link monitoring	2238

A.10.3.1.1	Introduction	2238

A.10.3.1.2	Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	2239

A.10.3.1.2.1	Test purpose and environment	2239

A.10.3.1.2.2	Test requirements	2244

A.10.3.1.3	Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	2244

A.10.3.1.3.1	Test purpose and environment	2244

A.10.3.1.3.2	Test requirements	2250

A.10.3.1.4	Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode	2250

A.10.3.1.4.1	Test purpose and environment	2250

A.10.3.1.4.2	Test requirements	2250

A.10.3.1.5	Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode	2250

A.10.3.1.5.1	Test purpose and environment	2250

A.10.3.1.5.2	Test requirements	2250

A.10.3.2	Interruption	2250

A.10.3.2.1	E-UTRAN – NR interruptions during SCell operations with CCA	2250

A.10.3.2.1.1	Test Purpose and Environment	2250

A.10.3.2.1.2	Test Requirements	2255

A.10.3.3	SCell activation and deactivation delay	2255

A.10.3.4	Beam failure detection and link recovery procedures	2262

A.10.3.5	Active BWP switching	2273

A.10.3.5.1	UL active BWP switch delay with consistent UL LBT failure on PSCell subject to UL CCA in EN-DC	2273

A.10.3.5.1.2	Test Requirements	2278

A.10.3.5.2	DCI-based and Timer-based Active BWP Switch	2279

A.10.3.5.2.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	2279

A.10.3.5.2.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	2282

A.10.3.5.3	RRC-based Active BWP Switch	2286

A.10.3.6	PSCell addition and release delay	2290

A.10.3.6.1	Addition and Release Delay of known NR PSCell on the carrier under CCA	2290

A.10.3.6.1.1	Test purpose and environment	2290

A.10.3.6.1.2	Test Requirements	2294

A.10.3.7	Void	2295

A.10.4	Measurement procedure	2295

A.10.4.1	Intra-frequency measurements	2295

A.10.4.1.1	Event-triggered reporting tests on PSCC without gaps under non-DRX	2295

A.10.4.1.1.1	Test purpose and environment	2295

A.10.4.1.1.2	Test parameters	2295

A.10.4.1.1.3	Test Requirements	2295

A.10.4.1.2	Event-triggered reporting tests on PSCC without gaps under DRX	2296

A.10.4.1.2.1	Test purpose and environment	2296

A.10.4.1.2.2	Test parameters	2296

A.10.4.1.2.3	Test Requirements	2296

A.10.4.1.3	Event-triggered reporting tests on PSCC with per-UE gaps under non-DRX	2296

A.10.4.1.3.1	Test purpose and environment	2296

A.10.4.1.3.2	Test parameters	2296

A.10.4.1.3.3	Test Requirements	2296

A.10.4.1.4	Event-triggered reporting tests on PSCC with per-UE gaps under DRX	2297

A.10.4.1.4.1	Test purpose and environment	2297

A.10.4.1.4.2	Test parameters	2297

A.10.4.1.4.3	Test Requirements	2297

A.10.4.1.5	Event-triggered reporting tests on SCC without gaps under non-DRX	2297

A.10.4.1.5.1	Test purpose and environment	2297

A.10.4.1.5.2	Test parameters	2297

A.10.4.1.5.3	Test Requirements	2297

A.10.4.1.6	Event-triggered reporting tests on SCC without gaps under DRX	2298

A.10.4.1.6.1	Test purpose and environment	2298

A.10.4.1.6.2	Test parameters	2298

A.10.4.1.6.3	Test Requirements	2298

A.10.4.1.7	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	2298

A.10.4.1.7.1	Test purpose and environment	2298

A.10.4.1.7.2	Test parameters	2298

A.10.4.1.7.3	Test Requirements	2298

A.10.4.1.8	Event-triggered reporting tests on SCC with per-UE gaps under DRX	2299

A.10.4.1.8.1	Test purpose and environment	2299

A.10.4.1.8.2	Test parameters	2299

A.10.4.1.8.3	Test Requirements	2299

A.10.4.1.9	RSSI measurement reporting on PSCC	2299

A.10.4.1.9.1	Test purpose and environment	2299

A.10.4.1.9.2	Test parameters	2299

A.10.4.1.10	Channel occupancy measurement reporting on PSCC	2300

A.10.4.1.10.1	Test purpose and environment	2300

A.10.4.1.10.2	Test parameters	2300

A.10.4.1.11	RSSI measurement reporting on SCC	2300

A.10.4.1.11.1	Test purpose and environment	2300

A.10.4.1.11.2	Test parameters	2300

A.10.4.1.12	Channel occupancy measurement reporting on SCC	2301

A.10.4.1.12.1	Test purpose and environment	2301

A.10.4.1.12.2	Test parameters	2301

A.10.4.2	Inter-frequency measurements	2301

A.10.4.2.1	RSSI measurement reporting	2301

A.10.4.2.1.1	Test purpose and environment	2301

A.10.4.2.1.2	Test parameters	2301

A.10.4.2.2	Channel occupancy measurement reporting	2302

A.10.4.2.2.1	Test purpose and environment	2302

A.10.4.2.2.2	Test parameters	2302

A.10.4.2.3	EN-DC event triggered reporting tests for FR1 with CCA cell without SSB time index detection when DRX is not used	2302

A.10.4.2.3.1	Test Purpose and Environment	2302

A.10.4.2.3.2	Test Requirements	2307

A.10.4.2.4	EN-DC event triggered reporting tests for FR1 cell with CCA without SSB time index detection when DRX is used	2308

A.10.4.2.4.1	Test Purpose and Environment	2308

A.10.4.2.4.2	Test Requirements	2312

A.10.4.2.5	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is not used	2313

A.10.4.2.5.1	Test Purpose and Environment	2313

A.10.4.2.5.2	Test Requirements	2317

A.10.4.2.6	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is used	2318

A.10.4.2.6.1	Test Purpose and Environment	2318

A.10.4.2.6.2	Test Requirements	2322

A.10.4.2.7	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	2323

A.10.4.2.7.1	Test Purpose and Environment	2323

A.10.4.2.7.2	Test Requirements	2329

A.10.4.2.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	2329

A.10.4.2.8.1	Test Purpose and Environment	2329

A.10.4.2.8.2	Test Requirements	2335

A.10.4.2.9	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	2336

A.10.4.2.9.1	Test Purpose and Environment	2336

A.10.4.2.9.2	Test Requirements	2341

A.10.4.2.10	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	2341

A.10.4.2.10.1	Test Purpose and Environment	2341

A.10.4.2.10.2	Test Requirements	2347

A.10.4.3	L1-RSRP measurements for beam reporting	2348

A.10.4.3.1	SSB based L1-RSRP measurement on PSCC when DRX is not used	2348

A.10.4.3.1.1	Test Purpose and Environment	2348

A.10.4.3.1.2	Test parameters	2348

A.10.4.3.1.3	Test Requirements	2350

A.10.4.3.2	SSB based L1-RSRP measurement on PSCC when DRX is used	2351

A.10.4.3.2.1	Test Purpose and Environment	2351

A.10.4.3.2.2	Test parameters	2351

A.10.4.3.2.3	Test Requirements	2353

A.10.4.3.3	SSB based L1-RSRP measurement on SCC when DRX is not used	2354

A.10.4.3.3.1	Test Purpose and Environment	2354

A.10.4.3.3.2	Test parameters	2354

A.10.4.3.3.3	Test Requirements	2357

A.10.4.3.4	SSB based L1-RSRP measurement on SCC when DRX is used	2358

A.10.4.3.4.1	Test Purpose and Environment	2358

A.10.4.3.4.2	Test parameters	2358

A.10.4.3.4.3	Test Requirements	2361

A.10.4.4	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	2362

A.10.4.4.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2362

A.10.4.4.1.1	Test Purpose and Environment	2362

A.10.4.4.1.2	Test Requirements	2366

A.10.4.4.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2367

A.10.4.4.2.1	Test Purpose and Environment	2367

A.10.4.4.2.2	Test Requirements	2372

A.10.4.4.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2373

A.10.4.4.3.1	Test Purpose and Environment	2373

A.10.4.4.3.2	Test Requirements	2377

A.10.4.4.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2378

A.10.4.4.4.1	Test Purpose and Environment	2378

A.10.4.4.4.2	Test Requirements	2383

A.10.5	Measurement performance	2384

A.10.5.1	SS-RSRP	2384

A.10.5.1.1	Intra-frequency measurement accuracy on a CCA serving cell	2384

A.10.5.1.1.1	Test Purpose and Environment	2384

A.10.5.1.1.2	Test parameters	2384

A.10.5.1.1.3	Test Requirements	2387

A.10.5.1.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	2387

A.10.5.1.2.1	Test Purpose and Environment	2387

A.10.5.1.2.2	Test parameters	2387

A.10.5.1.2.3	Test Requirements	2391

A.10.5.2	SS-RSRQ	2391

A.10.5.2.1	Intra-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	2391

A.10.5.2.1.1	Test Purpose and Environment	2391

A.10.5.2.1.2	Test Parameters	2391

A.10.5.2.1.3	Test Requirements	2394

A.10.5.2.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	2394

A.10.5.2.2.1	Test Purpose and Environment	2394

A.10.5.2.2.2	Test Parameters	2394

A.10.5.2.2.3	Test Requirements	2397

A.10.5.3	SS-SINR	2397

A.10.5.3.1	Intra-frequency measurement accuracy on PSCC	2397

A.10.5.3.1.1	Test Purpose and Environment	2397

A.10.5.3.1.2	Test Parameters	2397

A.10.5.3.1.3	Test Requirements	2400

A.10.5.3.2	Inter-frequency measurement accuracy on PSCC	2400

A.10.5.3.2.1	Test Purpose and Environment	2400

A.10.5.3.2.2	Test Parameters	2400

A.10.5.3.2.3	Test Requirements	2404

A.10.5.3.3	Intra-frequency measurement accuracy on SCC	2404

A.10.5.3.3.1	Test Purpose and Environment	2404

A.10.5.3.3.2	Test Parameters	2404

A.10.5.3.3.3	Test Requirements	2407

A.10.5.4	L1-RSRP measurement for beam reporting with CCA serving cell	2407

A.10.5.4.1	SSB based L1-RSRP measurement	2407

A.10.5.4.1.1	Test Purpose and Environment	2407

A.10.5.4.1.2	Test parameters	2408

A.10.5.4.1.3	Test Requirements	2410

A.10.5.5	RSSI	2410

A.10.5.5.1 	RSSI measurement accuracy on PSCC with CCA	2410

A.10.5.5.1.1	Test Purpose and Environment	2410

A.10.5.5.1.2	Test parameters	2410

A.10.5.5.1.3	Test Requirements	2413

A.10.5.5.2 	RSSI measurement accuracy on SCC with CCA	2413

A.10.5.5.2.1	Test Purpose and Environment	2413

A.10.5.5.2.2	Test parameters	2413

A.10.5.5.2.3	Test Requirements	2416

A.10.5.5.3 	Inter-frequency RSSI measurement accuracy on a carrier with CCA	2416

A.10.5.5.3.1	Test Purpose and Environment	2416

A.10.5.5.3.2	Test parameters	2416

A.10.5.5.3.3	Test Requirements	2420

A.10.5.6	Channel occupancy	2420

A.10.5.6.1 	Channel occupancy measurement accuracy on PSCC with CCA	2420

A.10.5.6.1.1	Test Purpose and Environment	2420

A.10.5.6.1.2	Test parameters	2420

A.10.5.6.1.3	Test Requirements	2424

A.10.5.6.2 	Channel occupancy measurement accuracy on SCC with CCA	2424

A.10.5.6.2.1	Test Purpose and Environment	2424

A.10.5.6.2.2	Test parameters	2424

A.10.5.6.2.3	Test Requirements	2427

A.10.5.6.3 	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	2427

A.10.5.6.3.1	Test Purpose and Environment	2427

A.10.5.6.3.2	Test parameters	2427

A.10.5.6.3.3	Test Requirements	2431

A.11	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	2431

A.11.1	RRC_IDLE state mobility	2431

A.11.1.1	Cell re-selection with both source and target NR carrier frequencies under CCA	2431

A.11.1.1.1	Cell reselection to FR1 intra-frequency NR cells when subject to CCA on the serving and target cell	2431

A.11.1.1.1.1	Test Purpose and Environment	2431

A.11.1.1.1.2	Test Parameters	2432

A.11.1.1.1.3	Test Requirements	2435

A.11.1.1.2	Cell reselection to FR1 inter-frequency NR case when subject to CCA on the serving and target cell	2435

A.11.1.1.2.1	Test Purpose and Environment	2435

A.11.1.1.2.2	Test Parameters	2435

A.11.1.1.2.3	Test Requirements	2439

A.11.1.2	Cell re-selection to NR with source NR carrier frequency under CCA	2439

A.11.1.2.1	Cell reselection to FR1 inter-frequency NR case when serving cell is subject to CCA	2439

A.11.1.2.1.1	Test Purpose and Environment	2439

A.11.1.2.1.2	Test Parameters	2440

A.11.1.2.1.3	Test Requirements	2446

A.11.1.3	Cell re-selection from NR carrier with target NR carrier frequency under CCA	2447

A.11.1.3.1	Cell reselection to FR1 inter-frequency NR case when target cell is subject to CCA	2447

A.11.1.3.1.1	Test Purpose and Environment	2447

A.11.1.3.1.2	Test Parameters	2447

A.11.1.3.1.3	Test Requirements	2452

A.11.1.4	Inter-RAT cell re-selection to E-UTRAN with source NR carrier frequency under CCA	2453

A.11.1.4.1	Cell reselection to higher priority E-UTRAN when serving cell is subject to CCA	2453

A.11.1.4.1.1	Test Purpose and Environment	2453

A.11.1.4.1.2	Test Parameters	2453

A.11.1.4.1.3	Test Requirements	2456

A.11.1.4.2	Cell reselection to lower priority E-UTRAN when serving cell is subject to CCA	2457

A.11.1.4.2.1	Test Purpose and Environment	2457

A.11.1.4.2.2	Test Requirements	2459

A.11.2	RRC_CONNECTED state mobility	2460

A.11.2.1	Handover	2460

A.11.2.1.1	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; known target cell	2460

A.11.2.1.1.1	Test Purpose and Environment	2460

A.11.2.1.1.2	Test Parameters	2460

A.11.2.1.1.3 Test Requirements	2463

A.11.2.1.2	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	2463

A.11.2.1.2.1	Test Purpose and Environment	2463

A.11.2.1.2.2	Test Parameters	2463

A.11.2.1.2.3	Test Requirements	2466

A.11.2.1.3	Inter-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	2466

A.11.2.1.3.1	Test Purpose and Environment	2466

A.11.2.1.3.2	Test Parameters	2466

A.11.2.1.3.3 Test Requirements	2469

A.11.2.1.4	Inter-frequency handover from FR1 carrier under CCA to FR1; known target cell	2469

A.11.2.1.4.1	Test Purpose and Environment	2469

A.11.2.1.4.2	Test Parameters	2469

A.11.2.1.4.3 Test Requirements	2473

A.11.2.1.5	Inter-frequency handover from FR1 carrier under CCA to FR1; unknown target cell	2474

A.11.2.1.5.1	Test Purpose and Environment	2474

A.11.2.1.5.2	Test Parameters	2474

A.11.2.1.5.3 Test Requirements	2477

A.11.2.1.6	Inter-frequency handover from FR1 to FR1 carrier under CCA; unknown target cell	2478

A.11.2.1.6.1	Test Purpose and Environment	2478

A.11.2.1.6.2	Test Parameters	2478

A.11.2.1.6.3 Test Requirements	2481

A.11.2.1.7	 SA NR FR1 carrier under CCA - E-UTRAN handover with known target cell	2482

A.11.2.1.7.1	Test Purpose and Environment	2482

A.11.2.1.7.2	Test Requirements	2488

A.11.2.1.8	SA NR FR1 carrier under CCA - E-UTRAN handover with unknown target cell	2488

A.11.2.1.8.1	Test Purpose and Environment	2488

A.11.2.1.8.2	Test Requirements	2493

A.11.2.2	RRC connection mobility control	2493

A.11.2.2.1	RRC re-establishment	2493

A.11.2.2.1.1	Intra-frequency RRC Re-establishment with CCA in FR1	2493

A.11.2.2.1.2	Inter-frequency RRC Re-establishment with CCA in FR1	2496

A.11.2.2.1.4	Inter-frequency RRC Re-establishment from NR FR1 carrier without CCA to NR FR1 carrier under CCA	2502

A.11.2.2.2	Random Access	2507

A.11.2.2.2.1	4-step RA type contention-based random access for NR PCell with CCA	2507

A.11.2.2.2.1.1	Test Purpose and Environment	2507

A.11.2.2.2.1.2	Test Requirements	2510

A.11.2.2.2.1.2.1	Random Access Preamble Transmission	2510

A.11.2.2.2.1.2.2	Random Access Response Reception	2510

A.11.2.2.2.1.2.3	No Random Access Response Reception	2511

A.11.2.2.2.1.2.4	Receiving an UL grant for msg3 retransmission	2511

A.11.2.2.2.1.2.5	Reception of an Incorrect Message over Temporary C-RNTI	2511

A.11.2.2.2.1.2.6	Reception of a Correct Message over Temporary C-RNTI	2512

A.11.2.2.2.1.2.7	Contention Resolution Timer expiry	2512

A.11.2.2.2.2	4-step RA type non-contention based random access for NR PSCell with CCA	2512

A.11.2.2.2.2.1	Test Purpose and Environment	2512

A.11.2.2.2.2.2	Test Requirements	2515

A.11.2.2.2.2.2.1	SSB-based Random Access Preamble Transmission	2515

A.11.2.2.2.2.2.2	Random Access Response Reception	2516

A.11.2.2.2.2.2.3	No Random Access Response Reception	2516

A.11.2.2.2.3	2-step RA type contention-based random access for NR PCell with CCA	2516

A.11.2.2.2.3.1	Test Purpose and Environment	2516

A.11.2.2.2.3.2	Test Requirements	2520

A.11.2.2.2.3.2.1	MsgA Transmission	2520

A.11.2.2.2.3.2.2	MsgB Reception	2520

A.11.2.2.2.3.2.3	No MsgB Reception	2521

A.11.2.2.2.4	2-step RA type non-contention-based random access for NR PCell with CCA	2521

A.11.2.2.2.4.1	Test Purpose and Environment	2521

A.11.2.2.2.4.2	Test Requirements	2525

A.11.2.2.2.4.2.1	MsgA Transmission	2525

A.11.2.2.2.4.2.2	MsgB Reception	2526

A.11.2.2.2.4.2.3	No MsgB Reception	2526

A.11.2.2.3	RRC connection release with redirection	2527

A.11.2.2.3.1	Redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA	2527

A.11.3	Timing	2535

A.11.3.1	UE transmit timing	2535

A.11.3.2	UE timing advance	2539

A.11.3.2.1	UE Timing Advance Adjustment Accuracy with PCell under DL CCA	2539

A.11.3.2.1.1	Test Purpose and Environment	2539

A.11.3.2.1.2	Test Parameters	2539

A.11.3.2.1.3	Test Requirements	2543

A.11.4	Signalling characteristics	2543

A.11.4.1	Radio link monitoring	2543

A.11.4.1.1	Introduction	2543

A.11.4.1.2	Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	2544

A.11.4.1.2.1	Test purpose and environment	2544

A.11.4.1.2.2	Test requirements	2548

A.11.4.1.3	Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	2548

A.11.4.1.3.1	Test purpose and environment	2548

A.11.4.1.3.2	Test requirements	2554

A.11.4.1.4	Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode	2554

A.11.4.1.4.1	Test purpose and environment	2554

A.11.4.1.4.2	Test requirements	2554

A.11.4.1.5	Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode	2554

A.11.4.1.5.1	Test purpose and environment	2554

A.11.4.1.5.2	Test requirements	2554

A.11.4.2	Interruption	2555

A.11.4.2.1	NR interruptions during Scell operations with CCA on PCell and SCell	2555

A.11.4.2.1.1 Test Purpose and Environment	2555

A.11.4.2.1.2	Test Requirements	2559

A.11.4.3	SCell activation and deactivation delay	2559

A.11.4.4	Beam failure detection and link recovery procedures	2566

A.11.4.4.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	2566

A.11.4.4.1.1	Test Purpose and Environment	2566

A.11.4.4.1.2	Test Requirements	2572

A.11.4.4.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	2572

A.11.4.4.2.1	Test Purpose and Environment	2572

A.11.4.4.2.2	Test Requirements	2578

A.11.4.5	Active BWP switching	2578

A.11.4.5.1	UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA	2578

A.11.4.5.1.2	Test Requirements	2582

A.11.4.6	Void	2592

A.11.5	Measurement procedure	2592

A.11.5.1	Intra-frequency measurements	2592

A.11.5.1.1	Event-triggered reporting tests on PCC without gaps under non-DRX	2592

A.11.5.1.1.1	Test purpose and environment	2592

A.11.5.1.1.2	Test parameters	2592

A.11.5.1.1.3	Test Requirements	2592

A.11.5.1.2	Event-triggered reporting tests on PCC without gaps under DRX	2593

A.11.5.1.2.1	Test purpose and environment	2593

A.11.5.1.2.2	Test parameters	2593

A.11.5.1.2.3	Test Requirements	2593

A.11.5.1.3	Event-triggered reporting tests on PCC with per-UE gaps under non-DRX	2593

A.11.5.1.3.1	Test purpose and environment	2593

A.11.5.1.3.2	Test parameters	2593

A.11.5.1.3.3	Test Requirements	2593

A.11.5.1.4	Event-triggered reporting tests on PCC with per-UE gaps under DRX	2594

A.11.5.1.4.1	Test purpose and environment	2594

A.11.5.1.4.2	Test parameters	2594

A.11.5.1.4.3	Test Requirements	2594

A.11.5.1.5	Event-triggered reporting tests on SCC without gaps under non-DRX	2594

A.11.5.1.5.1	Test purpose and environment	2594

A.11.5.1.5.2	Test parameters	2594

A.11.5.1.6	Event-triggered reporting tests on SCC without gaps under DRX	2595

A.11.5.1.6.1	Test purpose and environment	2595

A.11.5.1.6.2	Test parameters	2595

A.11.5.1.6.3	Test Requirements	2595

A.11.5.1.7	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	2595

A.11.5.1.7.1	Test purpose and environment	2595

A.11.5.1.7.2	Test parameters	2595

A.11.5.1.7.3	Test Requirements	2595

A.11.5.1.8	Event-triggered reporting tests on SCC with per-UE gaps under DRX	2596

A.11.5.1.8.1	Test purpose and environment	2596

A.11.5.1.8.2	Test parameters	2596

A.11.5.1.8.3	Test Requirements	2596

A.11.5.1.9	RSSI measurement reporting on PCC	2596

A.11.5.1.9.1	Test purpose and environment	2596

A.11.5.1.9.2	Test parameters	2596

A.11.5.1.10	Channel occupancy measurement reporting on PCC	2596

A.11.5.1.10.1	Test purpose and environment	2596

A.11.5.1.10.2	Test parameters	2597

A.11.5.1.11	RSSI measurement reporting on SCC	2597

A.11.5.1.11.1	Test purpose and environment	2597

A.11.5.1.11.2	Test parameters	2597

A.11.5.1.12	Channel occupancy measurement reporting on SCC	2597

A.11.5.1.12.1	Test purpose and environment	2597

A.11.5.1.12.2	Test parameters	2597

A.11.5.2	Inter-frequency measurements	2598

A.11.5.2.1	RSSI measurement reporting	2598

A.11.5.2.1.1	Test purpose and environment	2598

A.11.5.2.1.2	Test parameters	2598

A.11.5.2.2	Channel occupancy measurement reporting	2598

A.11.5.2.2.1	Test purpose and environment	2598

A.11.5.2.2.2	Test parameters	2598

A.11.5.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	2599

A.11.5.2.3.1	Test Purpose and Environment	2599

A.11.5.2.3.2	Test Requirements	2603

A.11.5.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	2604

A.11.5.2.4.1	Test Purpose and Environment	2604

A.11.5.2.4.2	Test Requirements	2609

A.11.5.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	2609

A.11.5.2.5.1	Test Purpose and Environment	2609

A.11.5.2.5.2	Test Requirements	2613

A.11.5.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	2614

A.11.5.2.6.1	Test Purpose and Environment	2614

A.11.5.2.6.2	Test Requirements	2619

A.11.5.2.7	Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2619

A.11.5.2.7.1	Test Purpose and Environment	2619

A.11.5.2.7.2	Test Requirements	2623

A.11.5.2.8	Event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2624

A.11.5.2.8.1	Test Purpose and Environment	2624

A.11.5.2.8.2	Test Requirements	2629

A.11.5.2.9	Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2629

A.11.5.2.9.1	Test Purpose and Environment	2629

A.11.5.2.9.2	Test Requirements	2634

A.11.5.2.10	Event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2635

A.11.5.2.10.1	Test Purpose and Environment	2635

A.11.5.2.10.2	Test Requirements	2640

A.11.5.3	Inter-RAT E-UTRAN measurements	2641

A.11.5.4	L1-RSRP measurements for beam reporting	2641

A.11.5.4.1	SSB based L1-RSRP measurement when DRX is not used	2641

A.11.5.4.1.1	Test Purpose and Environment	2641

A.11.5.4.1.2	Test parameters	2641

A.11.5.4.1.3	Test Requirements	2643

A.11.5.4.2	SSB based L1-RSRP measurement when DRX is used	2643

A.11.5.4.2.1	Test Purpose and Environment	2643

A.11.5.4.2.2	Test parameters	2644

A.11.5.4.2.3	Test Requirements	2646

A.11.5.4.3	SSB based L1-RSRP measurement on SCC when DRX is not used	2646

A.11.5.4.3.1	Test Purpose and Environment	2646

A.11.5.4.3.2	Test parameters	2647

A.11.5.4.3.3	Test Requirements	2650

A.11.5.4.4	SSB based L1-RSRP measurement on SCC when DRX is used	2651

A.11.5.4.4.1	Test Purpose and Environment	2651

A.11.5.4.4.2	Test parameters	2651

A.11.5.4.4.3	Test Requirements	2654

A.11.6	Measurement performance	2655

A.11.6.1	SS-RSRP	2655

A.11.6.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	2655

A.11.6.1.1.1	Test Purpose and Environment	2655

A.11.6.1.1.2	Test parameters	2655

A.11.6.1.1.3	Test Requirements	2657

A.11.6.1.2	Intra-frequency measurement accuracy on SCC on a carrier frequency with CCA	2657

A.11.6.1.2.1	Test Purpose and Environment	2657

A.11.6.1.2.2	Test parameters	2657

A.11.6.1.2.3	Test Requirements	2659

A.11.6.2	SS-RSRQ	2659

A.11.6.2.1	Intra-frequency measurement accuracy	2659

A.11.6.2.1.1	Test Purpose and Environment	2659

A.11.6.2.1.2	Test Parameters	2659

A.11.6.2.1.3	Test Requirements	2662

A.11.6.2.2	Inter-frequency measurement accuracy	2662

A.11.6.2.2.1	Test Purpose and Environment	2662

A.11.6.2.2.2	Test Parameters	2662

A.11.6.2.2.3	Test Requirements	2665

A.11.6.2.3	Intra-frequency measurement accuracy on SCC	2665

A.11.6.2.3.1	Test Purpose and Environment	2665

A.11.6.2.3.2	Test Parameters	2665

A.11.6.2.3.3	Test Requirements	2668

A.11.6.2.4	Inter-frequency measurement accuracy	2668

A.11.6.2.4.1	Test Purpose and Environment	2668

A.11.6.2.4.2	Test Parameters	2668

A.11.6.2.4.3	Test Requirements	2677

A.11.6.3	SS-SINR	2677

A.11.6.3.1	Intra-frequency measurement accuracy	2677

A.11.6.3.1.1	Test Purpose and Environment	2677

A.11.6.3.1.2	Test Parameters	2677

A.11.6.3.1.3	Test Requirements	2680

A.11.6.3.2	Inter-frequency measurement accuracy	2680

A.11.6.3.2.1	Test Purpose and Environment	2680

A.11.6.3.2.2	Test Parameters	2680

A.11.6.3.2.3	Test Requirements	2683

A.11.6.3.3	Intra-frequency measurement accuracy on SCC	2683

A.11.6.3.3.1	Test Purpose and Environment	2683

A.11.6.3.3.2	Test Parameters	2683

A.11.6.3.3.3	Test Requirements	2686

A.11.6.3.4	Inter-frequency measurement accuracy	2686

A.11.6.3.4.1	Test Purpose and Environment	2686

A.11.6.3.4.2	Test Parameters	2686

A.11.6.3.4.3	Test Requirements	2694

A.11.6.4	L1-RSRP measurement for beam reporting with CCA serving cell	2694

A.11.6.4.1	SSB based L1-RSRP measurement	2694

A.11.6.4.1.1	Test Purpose and Environment	2694

A.11.6.4.1.2	Test parameters	2695

A.11.6.4.1.3	Test Requirements	2698

A.11.6.5	RSSI	2698

A.11.6.5.1 	Intra-frequency RSSI measurement accuracy on PCC with CCA	2698

A.11.6.5.1.1	Test Purpose and Environment	2698

A.11.6.5.1.2	Test parameters	2698

A.11.6.5.1.3	Test Requirements	2701

A.11.6.5.2 	Intra-frequency RSSI measurement accuracy on SCC with CCA	2701

A.11.6.5.2.1	Test Purpose and Environment	2701

A.11.6.5.2.2	Test parameters	2701

A.11.6.5.2.3	Test Requirements	2704

A.11.6.5.3 	Inter-frequency RSSI measurement accuracy on a carrier with CCA	2704

A.11.6.5.3.1	Test Purpose and Environment	2704

A.11.6.5.3.2	Test parameters	2704

A.11.6.5.3.3	Test Requirements	2707

A.11.6.6	Channel occupancy	2707

A.11.6.6.1 	Intra-frequency channel occupancy measurement accuracy on PCC with CCA	2707

A.11.6.6.1.1	Test Purpose and Environment	2707

A.11.6.6.1.2	Test parameters	2707

A.11.6.6.1.3	Test Requirements	2711

A.11.6.6.2 	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	2711

A.11.6.6.2.1	Test Purpose and Environment	2711

A.11.6.6.2.2	Test parameters	2711

A.11.6.6.2.3	Test Requirements	2714

A.11.6.6.3 	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	2714

A.11.6.6.3.1	Test Purpose and Environment	2714

A.11.6.6.3.2	Test parameters	2714

A.11.6.6.3.3	Test Requirements	2717

A.11.6.7	E-UTRAN RSRP	2717

A.11.6.8	E-UTRAN RSRQ	2717

A.11.6.9	E-UTRAN SINR	2717

A.12	E-UTRA Standalone Tests with at Least One NR Cell under CCA	2717

A.12.1	RRC_IDLE state mobility	2717

A.12.1.1	Inter-RAT cell re-selection to NR on a carrier frequency with CCA	2717

A.12.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1 when target cell is subject to CCA	2717

A.12.1.1.1.1	Test Purpose and Environment	2717

A.12.1.1.1.2	Test Requirements	2723

A.12.2	RRC_CONNECTED state mobility	2724

A.12.2.1	Handover	2724

A.12.2.1.1	E-UTRAN - NR with CCA handover	2724

A.12.2.1.1.1	Test Purpose and Environment	2724

A.12.2.1.1.2	Test Requirements	2729

A.12.3	Signalling characteristics	2730

A.12.3.1	Interruptions	2730

A.12.4	Measurement procedure	2730

A.12.4.1	E-UTRANNR inter-RAT SFTD measurements	2730

A.12.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay with NR under CCA in non-DRX	2730

A.12.4.1.1.1	Test Purpose and Environment	2730

A.12.4.1.1.2	Test Requirements	2734

A.12.4.2	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	2734

A.12.4.2.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2734

A.12.4.2.1.1	Test Purpose and Environment	2734

A.12.4.2.1.2	Test Requirements	2738

A.12.4.2.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2739

A.12.4.2.2.1	Test Purpose and Environment	2739

A.12.4.2.2.2	Test Requirements	2743

A.12.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2743

A.12.4.2.3.1	Test Purpose and Environment	2743

A.12.4.2.3.2	Test Requirements	2747

A.12.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2748

A.12.4.2.4.1	Test Purpose and Environment	2748

A.12.4.2.4.2	Test Requirements	2752

A.12.4.2.5	RSSI measurement reporting	2752

A.12.4.2.5.1	Test purpose and environment	2752

A.12.4.2.5.2	Test parameters	2752

A.12.4.2.6	Channel occupancy measurement reporting	2753

A.12.4.2.6.1	Test purpose and environment	2753

A.12.4.2.6.2	Test parameters	2753

A.12.5	Measurement performance	2753

A.12.5.1	E-UTRANNR SFTD	2753

A.12.5.2	E-UTRANNR SS-RSRP	2759

A.12.5.3	E-UTRANNR SS-RSRQ	2759

A.12.5.4	E-UTRANNR SS-SINR	2759

A.12.5.5	E-UTRANNR RSSI	2759

A.12.5.6	E-UTRANNR channel occupancy	2759

A.13	 NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	2759

A.13.1	Timing	2759

A.13.1.1	UE transmit timing	2759

A.13.1.2	Timing advance	2759

A.13.2	Signalling characteristics	2759

A.13.2.1	Interruption	2759

A.13.2.1.1	NR interruptions during SCell operations with CCA on SCell	2759

A.13.2.1.1.1	Test Purpose and Environment	2759

A.13.2.1.1.2	Test Requirements	2763

A.13.2.2	SCell activation and deactivation delay	2763

A.13.2.3	Void	2770

A.13.3	Measurement procedure	2770

A.13.3.1	Intra-frequency measurements	2770

A.13.3.1.1	Event-triggered reporting tests on SCC without gaps under non-DRX	2770

A.13.3.1.1.1	Test purpose and environment	2770

A.13.3.1.1.2	Test parameters	2770

A.13.3.1.1.3	Test Requirements	2775

A.13.3.1.2	Event-triggered reporting tests on SCC without gaps under DRX	2775

A.13.3.1.2.1	Test purpose and environment	2775

A.13.3.1.2.2	Test parameters	2775

A.13.3.1.2.3	Test Requirements	2779

A.13.3.1.3	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	2779

A.13.3.1.3.1	Test purpose and environment	2779

A.13.3.1.3.2	Test parameters	2779

A.13.3.1.3.3	Test Requirements	2784

A.13.3.1.4	Event-triggered reporting tests on SCC with per-UE gaps under DRX	2784

A.13.3.1.4.1	Test purpose and environment	2784

A.13.3.1.4.2	Test parameters	2784

A.13.3.1.4.3	Test Requirements	2789

A.13.3.1.5	RSSI measurement reporting on SCC	2789

A.13.3.1.5.1	Test purpose and environment	2789

A.13.3.1.5.2	Test parameters	2789

A.13.3.1.6	Channel occupancy measurement reporting on SCC	2791

A.13.3.1.6.1	Test purpose and environment	2791

A.13.3.1.6.2	Test parameters	2792

A.13.3.2	Inter-frequency measurements	2792

A.13.3.2.1	RSSI measurement reporting	2792

A.13.3.2.1.1	Test purpose and environment	2792

A.13.3.2.1.2	Test parameters	2792

A.13.3.2.2	Channel occupancy measurement reporting	2793

A.13.3.2.2.1	Test purpose and environment	2793

A.13.3.2.2.2	Test parameters	2793

A.13.3.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	2793

A.13.3.2.3.1	Test Purpose and Environment	2793

A.13.3.2.3.2	Test Requirements	2799

A.13.3.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	2799

A.13.3.2.4.1	Test Purpose and Environment	2799

A.13.3.2.4.2	Test Requirements	2804

A.13.3.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	2805

A.13.3.2.5.1	Test Purpose and Environment	2805

A.13.3.2.5.2	Test Requirements	2810

A.13.3.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	2810

A.13.3.2.6.1	Test Purpose and Environment	2810

A.13.3.2.6.2	Test Requirements	2816

A.13.3.3	L1-RSRP measurements for beam reporting	2817

A.13.3.3.1	SSB based L1-RSRP measurement when DRX is not used	2817

A.13.3.3.1.1	Test Purpose and Environment	2817

A.13.3.3.1.2	Test parameters	2817

A.13.3.3.1.3	Test Requirements	2822

A.13.3.3.2	SSB based L1-RSRP measurement when DRX is used	2822

A.13.3.3.2.1	Test Purpose and Environment	2822

A.13.3.3.2.2	Test parameters	2822

A.13.3.3.2.3	Test Requirements	2826

A.13.4	Measurement performance	2826

A.13.4.1	SS-RSRP	2826

A.13.4.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	2826

A.13.4.1.1.1	Test Purpose and Environment	2826

A.13.4.1.1.2	Test parameters	2826

A.13.4.1.1.3	Test Requirements	2828

A.13.4.2	SS-RSRQ	2828

A.13.4.2.1	Intra-frequency measurement accuracy on SCC	2828

A.13.4.2.1.1	Test Purpose and Environment	2828

A.13.4.2.1.2	Test Parameters	2828

A.13.4.2.1.3	Test Requirements	2836

A.13.4.3	SS-SINR	2836

A.13.4.3.1	Intra-frequency measurement accuracy on SCC	2836

A.13.4.3.1.1	Test Purpose and Environment	2836

A.13.4.3.1.2	Test Parameters	2836

A.13.4.3.1.3	Test Requirements	2842

A.13.4.4	L1-RSRP measurement for beam reporting with CCA serving cell	2843

A.13.4.4.1	SSB based L1-RSRP measurement	2843

A.13.4.4.1.1	Test Purpose and Environment	2843

A.13.4.4.1.2	Test parameters	2843

A.13.4.4.1.3	Test Requirements	2847

A.13.4.5	RSSI	2847

A.13.4.5.1 	Intra-frequency RSSI measurement accuracy on a carrier with CCA	2847

A.13.4.5.1.1	Test Purpose and Environment	2847

A.13.4.5.1.2	Test parameters	2847

A.13.4.5.1.3	Test Requirements	2851

A.13.4.5.2	  Inter-frequency RSSI measurement accuracy on a carrier with CCA	2851

A.13.4.5.2.1	Test Purpose and Environment	2851

A.13.4.5.2.2	Test parameters	2851

A.13.4.5.2.3	Test Requirements	2855

A.13.4.6	Channel occupancy	2855

A.13.4.6.1 	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	2855

A.13.4.6.1.1	Test Purpose and Environment	2855

A.13.4.6.1.2	Test parameters	2855

A.13.4.6.1.3	Test Requirements	2859

A.13.4.6.2	  Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	2859

A.13.4.6.2.1	Test Purpose and Environment	2859

A.13.4.6.2.2	Test parameters	2859

A.13.4.6.2.3	Test Requirements	2863

Annex B (normative): Conditions for RRM requirements applicability for operating bands	1818

B.1	Conditions for NR RRC_IDLE state mobility	1818

B.1.1	Introduction	1818

B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection	1818

B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection	1819

B.2	Conditions for UE measurements procedures and performance requirements in RRC_CONNECTED state	1820

B.2.1	Introduction	1820

B.2.1.1	General	1820

B.2.1.2	Derivation of Minimum SSB_RP values for FR1	1820

B.2.1.3	Derivation of Minimum SSB_RP values for FR2	1820

B.2.1.3.1	Minimum SSB_RP values for Rx Beam Peak angle of arrival	1820

B.2.1.3.2	Minimum SSB_RP values for angle of arrival within Spherical coverage	1821

B.2.1.4	Gain to SS-RSRP and CSI-RSRP measurement point for FR1	1821

B.2.1.5	Gain to SS-RSRP and CSI-RSRP measurement point for FR2	1822

B.2.1.5.1	Gain to SS-RSRP and CSI-RSRP measurement point for Rx Beam Peak angle of arrival	1822

B.2.1.6	Gain to PRS-RSRP measurement point for FR2	1822

B.2.1.6.1	Gain to PRS-RSRP measurement point for Rx Beam Peak angle of arrival	1822

B.2.2	Conditions for NR intra-frequency measurements	1823

B.2.3	Conditions for NR inter-frequency measurements	1824

B.2.4	Conditions for NR L1-RSRP reporting	1826

B.2.4.1	Conditions for SSB based L1-RSRP reporting	1826

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting	1827

B.2.5	Conditions for RRC connection release with redirection to NR	1828

B.2.6	Void	1829

B.2.6.1	Void	1829

B.2.6.2	Void	1830

B.2.7	Conditions for SRS-RSRP measurements	1830

B.2.8	Conditions for NR L1-SINR reporting	1830

B.2.8.1	Conditions for L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	1830

B.2.8.2	Conditions for L1-SINR reporting with SSB based CMR and dedicated IMR configured	1831

B.2.8.2.1	L1-SINR reporting with SSB based CMR and dedicated ZP-IMR configured	1831

B.2.8.2.2	L1-SINR reporting with SSB based CMR and dedicated NZP-IMR configured	1832

B.2.8.3	Conditions for L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	1834

B.2.8.3.1	L1-SINR reporting with CSI-RS based CMR and dedicated ZP-IMR configured	1834

B.2.8.3.2	L1-SINR reporting with CSI-RS based CMR and dedicated NZP-IMR configured	1835

B.2.9	Conditions for NR intra-frequency measurements under CCA	1836

B.2.10	Conditions for NR inter-frequency measurements under CCA	1836

B.2.11	Conditions for NR L1-RSRP reporting under CCA	1836

B.2.11.1	Conditions for SSB based L1-RSRP reporting	1836

B.2.12	Conditions for NR CSI-RS based intra-frequency measurements	1837

B.2.13	Conditions for NR CSI-RS based inter-frequency measurements	1838

B.2.14	Conditions for NR PRS-based measurements	1840

B.3	RRM Requirements Exceptions	1841

B.3.1	Introduction	1841

B.3.2	Receiver sensitivity relaxation for CA	1841

B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1	1841

B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1	1842

B.3.2.2.1	Inter-band carrier aggregation	1842

B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA	1842

B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	1842

B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2	1842

B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2	1842

B.3.2.4.1	Intra-band contiguous carrier aggregation	1842

B.3.2.4.2	Intra-band non-contiguous carrier aggregation	1843

B.3.3	Receiver sensitivity relaxation for DC	1843

B.3.3.1	Receiver sensitivity relaxation for EN-DC	1843

B.3.3.2	Receiver sensitivity relaxation for NE-DC	1843

B.3.4	Receiver sensitivity relaxation for SUL	1843

B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1	1843

B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1	1843

B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL	1843

B.4	Conditions for V2X	1843

B.4.1	Test parameters for GNSS signals	1843

B.4.2	Conditions for PSBCH-RSRP Accuracy Requirements	1844

B.4.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	1844

B.4.4	Conditions for L1 SL-RSRP Accuracy Requirements	1844

Annex C (informative): Change history	1846