| 3GPP TS 38.133 V17.2.0 (2021-06) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Requirements for support of radio resource management (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	42

1	Scope	44

2	References	44

3	Definitions, symbols and abbreviations	45

3.1	Definitions	45

3.2	Symbols	46

3.3	Abbreviations	47

3.4	Test tolerances	50

3.5	Frequency bands grouping	50

3.5.1	Introduction	50

3.5.2	NR operating bands in FR1	50

3.5.3	NR operating bands in FR2	51

3.6	Applicability of requirements in this specification version	52

3.6.1	RRC connected state requirements in DRX	53

3.6.2	Number of serving carriers	53

3.6.2.1	Number of serving carriers for SA	53

3.6.2.2	Number of serving carriers for EN-DC	53

3.6.2.3	Number of serving carriers for NE-DC	53

3.6.2.4	Number of serving carriers for NR-DC	54

3.6.3	Applicability for intra-band FR2	54

3.6.4	Applicability for FR2 UE power classes	54

3.6.5	Applicability for SDL bands	54

3.6.6	Applicability of requirements for NGEN-DC operation	54

3.6.7	Applicability of QCL	54

3.6.9	Applicability of requirements for scheduling availability	55

3.6.10	Applicability of requirements for measurement restrictions	55

4	SA: RRC_IDLE state mobility	55

4.1	Cell Selection	55

4.2	Cell Re-selection	55

4.2.1	Introduction	55

4.2.2	Requirements	56

4.2.2.1	UE measurement capability	56

4.2.2.2	Measurement and evaluation of serving cell	56

4.2.2.3	Measurements of intra-frequency NR cells	56

4.2.2.4	Measurements of inter-frequency NR cells	58

4.2.2.5	Measurements of inter-RAT E-UTRAN cells	60

4.2.2.6	Maximum interruption in paging reception	62

4.2.2.7	General requirements	62

4.2.2.8	Minimum requirement at transitions	63

4.2.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	63

4.2.2.9.1	Introduction	63

4.2.2.9.2	Measurements for UE fulfilling low mobility criterion	63

4.2.2.9.3	Measurements for UE fulfilling not-at-cell edge criterion	64

4.2.2.9.4	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	64

4.2.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	65

4.2.2.10.1	Introduction	65

4.2.2.10.2	Measurements for UE fulfilling low mobility criterion	65

4.2.2.10.3	Measurements for UE fulfilling not-at-cell edge criterion	66

4.2.2.10.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	66

4.2.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	67

4.2.2.11.2	Measurements for UE fulfilling low mobility criterion	67

4.2.2.11.3	Measurements for UE fulfilling with not-at-cell edge criterion	68

4.2.2.11.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	68

4.2A	Cell Re-selection when subject to CCA	69

4.2A.1	Introduction	69

4.2A.2	Requirements	69

4.2A.2.1	UE measurement capability	69

4.2A.2.2	Measurement and evaluation when subject to CCA on the serving cell	69

4.2A.2.3	Measurements of intra-frequency NR cells when subject to CCA on the serving cell and target cell	70

4.2A.2.4	Measurements of inter-frequency NR cells when subject to CCA on the target cell	71

4.2A.2.5	Measurements of inter-RAT E-UTRAN cells when subject to CCA on the serving cell	73

4.2A.2.6	Maximum interruption in paging reception when subject to CCA on the target cell	73

4.2A.2.7	General requirements	73

4.3	Minimization of Drive Tests (MDT)	74

4.3.1	Introduction	74

4.3.2	Measurement Requirements	74

4.3.3	Requirements for Relative Time Stamp Accuracy	74

4.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	75

4.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	75

4.4	Idle Mode CA/DC Measurements	75

4.4.1	Introduction	75

4.4.2	Measurement Requirements	75

4.4.2.1	Detected cell requirement during state transition and Idle mode	75

4.4.2.2	Measurements of inter-frequency CA/DC candidate cells	76

4.4.2.3	Measurements on serving cell	77

4.4.2.4	Measurements of E-UTRAN inter-RAT DC candidate cells	77

5	SA: RRC_INACTIVE state mobility	79

5.1	Cell Re-selection	79

5.1.1	Introduction	79

5.1.2	Requirements	79

5.1.2.1	UE measurement capability	79

5.1.2.2	Measurement and evaluation of serving cell	79

5.1.2.3	Measurements of intra-frequency NR cells	79

5.1.2.4	Measurements of inter-frequency NR cells	79

5.1.2.5	Measurements of inter-RAT E-UTRAN cells	79

5.1.2.6	Maximum interruption in paging reception	79

5.1.2.7	General requirements	79

5.1A	Cell Re-selection with CCA	79

5.1A.1	Introduction	79

5.1A.2	Requirements	80

5.1A.2.1	UE measurement capability	80

5.1A.2.2	Measurement and evaluation when CCA is used on the serving cell	80

5.1A.2.3	Measurements of intra-frequency NR cells when CCA is used on the serving cell and target cell	80

5.1A.2.4	Measurements of inter-frequency NR cells when CCA is used on the target cell	80

5.1A.2.5	Measurements of inter-RAT E-UTRAN cells when CCA is used on the serving cell	80

5.1A.2.6	Maximum interruption in paging reception when CCA is used on the target cell	80

5.1A.2.7	General requirements	80

5.2	Void	80

5.3	Minimization of Drive Tests (MDT)	80

5.3.1	Introduction	80

5.3.2	Measurement Requirements	81

5.3.3	Requirements for Relative Time Stamp Accuracy	81

5.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	81

5.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	81

5.3.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	81

5.4	Idle Mode CA/DC Measurements	81

5.4.1	Introduction	81

5.4.2	Measurement Requirements	81

5.4.2.1	Detected cell requirement during state transition and Idle mode	82

5.4.2.2	Measurements of inter-frequency CA/DC candidate cells	82

5.4.2.3	Measurements on serving cell	82

5.4.2.4	Measurements on E-UTRAN inter-RAT DC candidate cells	82

6	RRC_CONNECTED state mobility	82

6.1	Handover	82

6.1.1	NR Handover	82

6.1.1.1	Introduction	82

6.1.1.2	NR FR1 - NR FR1 Handover	82

6.1.1.2.1	Handover delay	82

6.1.1.2.2	Interruption time	82

6.1.1.3	NR FR2- NR FR1 Handover	83

6.1.1.3.1	Handover delay	83

6.1.1.3.2	Interruption time	83

6.1.1.4	NR FR2- NR FR2 Handover	84

6.1.1.4.1	Handover delay	84

6.1.1.4.2	Interruption time	84

6.1.1.5	NR FR1- NR FR2 Handover	85

6.1.1.5.1	Handover delay	85

6.1.1.5.2	Interruption time	85

6.1.2	NR Handover to other RATs	86

6.1.2.1	NR – E-UTRAN Handover	86

6.1.2.1.1	Introduction	86

6.1.2.1.2	Handover delay	86

6.1.2.1.3	Interruption time	86

6.1.2.2	NR – UTRAN Handover	86

6.1.2.2.1	Introduction	86

6.1.2.2.2	Handover delay	87

6.1.2.2.3	Interruption time	87

6.1.3	NR DAPS Handover	87

6.1.3.1	Introduction	87

6.1.3.2	NR FR1 - NR FR1 DAPS Handover	88

6.1.3.2.1	DAPS handover delay	88

6.1.3.2.2	Interruption time	89

6.1.3.3	NR FR2- NR FR1 DAPS Handover	90

6.1.3.3.1	DAPS handover delay	91

6.1.3.3.2	Interruption time	91

6.1.3.4	NR FR1- NR FR2 DAPS Handover	92

6.1.3.4.1	DAPS handover delay	92

6.1.3.4.2	Interruption time	92

6.1.4	NR Conditional Handover	93

6.1.4.1	Introduction	93

6.1.4.2	NR FR1 – NR FR1 conditional handover	93

6.1.4.3	NR FR2 – NR FR1 conditional handover	94

6.1.4.4	NR FR2 – NR FR2 conditional handover	94

6.1.4.4.1	Handover delay	94

6.1.4.4.2	Measurement time	95

6.1.4.4.3	Preparation time	95

6.1.4.4.4	Interruption time	95

6.1.4.5	NR FR1 – NR FR2 conditional handover	96

6.1A	Void	96

6.1A.1	Void	96

6.1A.1.1	Void	96

6.1A.1.2	Void	96

6.1A.1.2.1	Void	96

6.1A.1.2.2	Void	96

6.1B	Handover to target cell using CCA	96

6.1B.1	NR Handover	96

6.1B.1.1	Introduction	96

6.1B.1.2	NR FR1 - NR FR1 Handover	96

6.1B.1.2.1	Handover delay	96

6.1B.1.2.2	Interruption time	97

6.2	RRC Connection Mobility Control	97

6.2.1	SA: RRC Re-establishment	97

6.2.1.1	Introduction	97

6.2.1.2	Requirements	98

6.2.1.2.1	UE Re-establishment delay requirement	98

6.2.1A	RRC Re-establishment with CCA	99

6.2.1A.1	Introduction	99

6.2.1A.2	Requirements	99

6.2.1A.2.1	UE Re-establishment with CCA delay requirement	100

6.2.2	Random access	101

6.2.2.1	Introduction	101

6.2.2.2	Requirements for 4-step RA type	101

6.2.2.2.1	Contention based random access	102

6.2.2.2.2	Non-Contention based random access	103

6.2.2.2.3	UE behaviour when configured with supplementary UL	104

6.2.2.3	Requirements for 2-step RA type	104

6.2.2.3.1	Contention based random access	104

6.2.2.3.2	Non-Contention based random access	105

6.2.2.3.3	UE behaviour when configured with supplementary UL	105

6.2.2A	Random access when CCA is used on target frequency	106

6.2.2A.1	Introduction	106

6.2.2A.2	Requirements for 4-step RA type	106

6.2.2A.2.1	Contention based random access	106

6.2.2A.2.2	Non-Contention based random access	107

6.2.2A.3	Requirements for 2-step RA type	108

6.2.2A.3.1	Contention based random access	109

6.2.2A.3.2	Non-Contention based random access	110

6.2.3	SA: RRC Connection Release with Redirection	110

6.2.3.1	Introduction	110

6.2.3.2	Requirements	111

6.2.3.2.1	RRC connection release with redirection to NR	111

6.2.3.2.2	RRC connection release with redirection to E-UTRAN	112

6.2.3.2.3	RRC connection release with redirection to NR carrier subject to CCA	112

7	Timing	113

7.1	UE transmit timing	113

7.1.1	Introduction	113

7.1.2	Requirements	114

7.1.2.1	Gradual timing adjustment	115

7.1.2.2	Void	115

7.2	UE timer accuracy	115

7.2.1	Introduction	115

7.2.2	Requirements	116

7.3	Timing advance	116

7.3.1	Introduction	116

7.3.2	Requirements	116

7.3.2.1	Timing Advance adjustment delay	116

7.3.2.2	Timing Advance adjustment accuracy	116

7.4	Cell phase synchronization accuracy	116

7.4.1	Definition	116

7.4.2	Minimum requirements	116

7.5	Maximum Transmission Timing Difference	117

7.5.1	Introduction	117

7.5.2	Minimum Requirements for inter-band EN-DC	117

7.5.2.1	Minimum Requirements for inter-band synchronous EN-DC	117

7.5.3	Minimum Requirements for intra-band EN-DC	118

7.5.4	Minimum Requirements for NR Carrier Aggregation	118

7.5.5	Minimum Requirements for inter-band NE-DC	118

7.5.5.1	Minimum Requirements for inter-band synchronous NE-DC	119

7.5.6	Minimum Requirements for inter-band NR DC	119

7.6	Maximum Receive Timing Difference	120

7.6.1	Introduction	120

7.6.2	Minimum Requirements for inter-band EN-DC	120

7.6.2.1	Minimum Requirements for inter-band synchronous EN-DC	120

7.6.3	Minimum Requirements for intra-band EN-DC	121

7.6.4	Minimum Requirements for NR Carrier Aggregation	121

7.6.5	Minimum Requirements for inter-band NE-DC	122

7.6.5.1	Minimum Requirements for inter-band synchronous NE-DC	122

7.6.6	Minimum Requirements for inter-band NR DC	123

7.7	deriveSSB-IndexFromCell tolerance	123

7.7.1	Minimum requirements	123

7.8	Void	124

8	Signalling characteristics	124

8.1	Radio Link Monitoring	124

8.1.1	Introduction	124

8.1.2	Requirements for SSB based radio link monitoring	125

8.1.2.1	Introduction	125

8.1.2.2	Minimum requirement	126

8.1.2.3	Measurement restrictions for SSB based RLM	127

8.1.3	Requirements for CSI-RS based radio link monitoring	128

8.1.3.1	Introduction	128

8.1.3.2	Minimum requirement	129

8.1.3.3	Measurement restrictions for CSI-RS based RLM	131

8.1.4	Minimum requirement at transitions	131

8.1.5	Minimum requirement for UE turning off the transmitter	132

8.1.6	Minimum requirement for L1 indication	132

8.1.7	Scheduling availability of UE during radio link monitoring	132

8.1.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	132

8.1.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	132

8.1.7.3	Scheduling availability of UE performing radio link monitoring on FR2	133

8.1.7.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	133

8.1A	Radio Link Monitoring with CCA on Target Frequency	133

8.1A.1	Introduction	133

8.1A.2	Requirements for SSB Based Radio Link Monitoring	134

8.1A.2.1	Introduction	134

8.1A.2.2	Minimum Requirement	135

8.1A.3	Minimum requirement at transitions	136

8.1A.4	Minimum requirement for UE turning off the transmitter	136

8.1A.5	Minimum requirement for L1 indication	137

8.1A.6	Scheduling availability of UE during radio link monitoring	137

8.1A.6.1	Scheduling availability of UE performing radio link monitoring with the same subcarrier spacing as PDSCH/PDCCH	137

8.1A.6.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH	137

8.2	Interruption	137

8.2.1	EN-DC Interruption	137

8.2.1.1	Introduction	137

8.2.1.2	Requirements	138

8.2.1.2.1	Interruptions at transitions between active and non-active during DRX	138

8.2.1.2.2	Interruptions at transitions from non-DRX to DRX	138

8.2.1.2.3	Interruptions at SCell addition/release	139

8.2.1.2.4	Interruptions at SCell activation/deactivation	140

8.2.1.2.5	Interruptions during measurements on SCC	141

8.2.1.2.6	Interruptions at UL carrier RRC reconfiguration	142

8.2.1.2.7	Interruptions due to Active BWP switching Requirement	143

8.2.1.2.8	Interruptions at direct SCell activation and hibernation	144

8.2.1.2.9	Interruptions at SCell hibernation	144

8.2.1.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	144

8.2.1.2.11	Interruptions due to UE-specific CBW change	145

8.2.1.2.12	 Interruptions at NR SRS carrier based switching	145

8.2.1.2.13	 Interruptions at E-UTRA SRS carrier based switching	146

8.2.1.2.14	DL Interruptions at switching between two uplink carriers	147

8.2.1.2.15	Interruptions due to SCell dormancy	148

8.2.1.2.16	 Interruptions when identifying CGI of an NR cell with autonomous gaps	148

8.2.1.2.17	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	149

8.2.2	SA: Interruptions with Standalone NR Carrier Aggregation	149

8.2.2.1	Introduction	149

8.2.2.2	Requirements	150

8.2.2.2.1	Interruptions at SCell addition/release	150

8.2.2.2.2	Interruptions at SCell activation/deactivation	151

8.2.2.2.3	Interruptions during measurements on deactivated SCC	152

8.2.2.2.4	Interruptions at UL carrier RRC reconfiguration	153

8.2.2.2.5	Interruptions due to Active BWP switching Requirement	153

8.2.2.2.6	Interruptions at inter-frequency SFTD measurement	154

8.2.2.2.7	Interruptions at SCell activation/deactivation with multiple downlink SCells	155

8.2.2.2.8	Interruptions due to UE-specific CBW change	155

8.2.2.2.9	 Interruptions at NR SRS carrier based switching	156

8.2.2.2.10	DL Interruptions at UE switching between two uplink carriers	157

8.2.2.2.11	Interruptions at direct SCell activation	158

8.2.2.2.12	Interruptions due to SCell dormancy	158

8.2.2.2.13	Interruptions at transitions between active and non-active during DRX	158

8.2.2.2.14	 Interruptions when identifying CGI of an NR cell with autonomous gaps	159

8.2.2.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	159

8.2.3	NE-DC Interruptions	160

8.2.3.1	Introduction	160

8.2.3.2	Requirements	160

8.2.3.2.1	Interruptions at transitions between active and non-active during DRX	160

8.2.3.2.2	Interruptions at transitions from non-DRX to DRX	161

8.2.3.2.3	Interruptions at PSCell/SCell addition/release	161

8.2.3.2.4	Interruptions at SCell activation/deactivation	162

8.2.3.2.5	Interruptions during measurements on SCC	163

8.2.3.2.6	Interruptions at UL carrier RRC reconfiguration	164

8.2.3.2.7	Interruptions due to Active BWP switching Requirement	165

8.2.3.2.8	Interruptions at direct SCell activation and hibernation	165

8.2.3.2.9	Interruptions at SCell hibernation	165

8.2.3.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	166

8.2.3.2.11	 Interruptions at NR SRS carrier based switching	166

8.2.3.2.12	 Interruptions at E-UTRA SRS carrier based switching	167

8.2.3.2.13	Interruptions due to SCell dormancy	168

8.2.3.2.14	 Interruptions when identifying CGI of an NR cell with autonomous gaps	169

8.2.3.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	169

8.2.4	NR-DC: Interruptions	170

8.2.4.1	Introduction	170

8.2.4.2	Requirements	170

8.2.4.2.1	Interruptions at PSCell/SCell addition/release	170

8.2.4.2.2	Interruptions at SCell activation/deactivation	171

8.2.4.2.3	Interruptions during measurements on SCC	172

8.2.4.2.4	Interruptions at UL carrier RRC reconfiguration	172

8.2.4.2.5	Interruptions due to Active BWP switching Requirement	173

8.2.4.2.6	Interruptions at transitions between active and non-active during DRX	173

8.2.4.2.7	Interruptions at transitions from non-DRX to DRX	173

8.2.4.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	173

8.2.4.2.9	Interruptions at NR SRS carrier based switching	174

8.2.4.2.10	Interruptions at direct SCell activation	175

8.2.4.2.11	 Interruptions when identifying CGI of an NR cell with autonomous gaps	175

8.2.4.2.12	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	176

8.2.4.2.13	 Interruptions due to SCell dormancy	177

8.2.4.2A	Void	177

8.2.4.2A.1	Void	177

8.2.4.2A.2	Void	177

8.2.4.2A.3	Void	177

8.3	SCell Activation and Deactivation Delay	177

8.3.1	Introduction	177

8.3.2	SCell Activation Delay Requirement for Deactivated SCell	177

8.3.3	SCell Deactivation Delay Requirement for Activated SCell	182

8.3.4	Direct SCell Activation at SCell addition	182

8.3.5	Direct SCell Activation at Handover	183

8.3.7	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	185

8.3.8	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	188

8.3.9	Direct SCell Activation of Multiple Downlink SCells at SCell addition	189

8.3.10	Direct SCell Activation of Multiple Downlink SCells at Handover	190

8.3A	SCell Activation and Deactivation Delay in Carriers with CCA	191

8.3A.1	Introduction	191

8.3A.2	SCell Activation Delay Requirement for Deactivated SCell	192

8.3A.3	SCell Deactivation Delay Requirement for Activated SCell	194

8.4	UE UL carrier RRC reconfiguration delay	194

8.4.1	Introduction	194

8.4.2	UE UL carrier configuration delay requirement	195

8.4.3	UE UL carrier deconfiguration delay requirement	195

8.5	Link Recovery Procedures	195

8.5.1	Introduction	195

8.5.2	Requirements for SSB based beam failure detection	196

8.5.2.1	Introduction	196

8.5.2.2	Minimum requirement	196

8.5.2.3	Measurement restriction for SSB based beam failure detection	198

8.5.3	Requirements for CSI-RS based beam failure detection	198

8.5.3.1	Introduction	198

8.5.3.2	Minimum requirement	199

8.5.3.3	Measurement restrictions for CSI-RS beam failure detection	201

8.5.4	Minimum requirement for L1 indication	202

8.5.5	Requirements for SSB based candidate beam detection	202

8.5.5.1	Introduction	202

8.5.5.2	Minimum requirement	202

8.5.5.3	Measurement restriction for SSB based candidate beam detection	204

8.5.6	Requirements for CSI-RS based candidate beam detection	205

8.5.6.1	Introduction	205

8.5.6.2	Minimum requirement	205

8.5.6.3	Measurement restriction for CSI-RS based candidate beam detection	207

8.5.7	Scheduling availability of UE during beam failure detection	208

8.5.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	208

8.5.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	208

8.5.7.3	Scheduling availability of UE performing beam failure detection on FR2	208

8.5.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR DC	209

8.5.8	Scheduling availability of UE during candidate beam detection	209

8.5.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	209

8.5.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	209

8.5.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	209

8.5.8.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	210

8.5.9	Requirements for Beam Failure Recovery in SCell	210

8.5.9.1	Introduction	210

8.5.9.2	Requirement	210

8.5A	Link Recovery Procedures when CCA is used on target frequency	211

8.5A.1	Introduction	211

8.5A.2	Requirements for SSB based beam failure detection	211

8.5A.2.1	Introduction	211

8.5A.2.2	Minimum requirement	212

8.5A.2.3	Measurement restriction for SSB based beam failure detection	212

8.5A.4	Minimum requirement for L1 indication	213

8.5A.5	Requirements for SSB based candidate beam detection	213

8.5A.5.1	Introduction	213

8.5A.5.2	Minimum requirement	213

8.5A.5.3	Measurement restriction for SSB based candidate beam detection	214

8.5A.7	Scheduling availability of UE during beam failure detection	214

8.5A.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH	214

8.5A.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH	214

8.5A.8	Scheduling availability of UE during candidate beam detection	214

8.5A.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH	214

8.5A.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH	215

8.6	Active BWP switch delay	215

8.6.1	Introduction	215

8.6.2	DCI and timer based BWP switch delay on a single CC	215

8.6.2A	DCI based BWP switch delay on multiple CCs	216

8.6.2A.1	Simultaneous DCI based BWP switch delay on multiple CCs	216

8.6.2A.2	Non-simultaneous DCI based BWP switch delay on multiple CCs	218

8.6.2B	Timer based BWP switch delay on multiple CCs	218

8.6.2B.1	Simultaneous timer based BWP switch delay on multiple CCs	218

8.6.2B.2	Non-simultaneous timer based BWP switch delay on multiple CCs	219

8.6.3	RRC based BWP switch delay on a single CC	219

8.6.3A	RRC based BWP switch delay on multiple CCs	220

8.6.3A.1	Simultaneous RRC based BWP switch delay on multiple CCs	220

8.6.3A.2	Non-simultaneous RRC based BWP switch delay on multiple CCs	220

8.6.4	BWP switch delay on Consistent UL CCA recovery	221

8.7	Void	221

8.8	NE-DC: E-UTRAN PSCell Addition and Release Delay	221

8.8.1	Introduction	221

8.8.2	E-UTRAN PSCell Addition Delay Requirement	221

8.8.3	E-UTRAN PSCell Release Delay Requirement	222

8.9	NR-DC: PSCell Addition and Release Delay	222

8.9.1	Introduction	222

8.9.2	PSCell Addition Delay Requirement	222

8.9.3	PSCell Release Delay Requirement	223

8.10	Active TCI state switching delay	223

8.10.4	DCI based TCI state switch delay	224

8.10.5	RRC based TCI state switch delay	224

8.10.6	Active TCI state list update delay	225

8.10A	Active TCI state switching delay with CCA	225

8.10A.1	Introduction	225

8.10A.2	Known conditions for TCI state	225

8.10A.3	MAC-CE based TCI state switch delay	226

8.10A.4	DCI based TCI state switch delay	226

8.10A.5	RRC based TCI state switch delay	227

8.10A.6	Active TCI state list update delay	227

8.11	PSCell Change	227

8.11A	void	228

8.11B	Conditional PSCell Change	228

8.11B.1	Introduction	228

8.11B.2	Conditoinal PSCell Change delay	228

8.11B.2.1	Measurement time	228

8.12	Uplink spatial relation switch delay	229

8.12.1	Introduction	229

8.12.2	Known conditions for spatial relation when associated with DL-RS	229

8.12.3	MAC-CE based spatial relation switch delay	229

8.12.4	DCI based spatial relation switch delay	230

8.12.5	RRC based spatial relation switch delay	230

8.13	UE-specific CBW change	230

8.13.1	Introduction	230

8.13.2	UE-specific CBW change delay	231

8.14	Pathloss reference signal switching delay	231

8.14.1	Introduction	231

8.14.2	Known conditions for pathloss reference signal	231

8.14.3	MAC-CE based pathloss reference signal switch delay	231

9	Measurement Procedure	232

9.1	General measurement requirement	232

9.1.1	Introduction	232

9.1.2	Measurement gap	232

9.1.2.1	EN-DC: Measurement Gap Sharing	243

9.1.2.1a	SA: Measurement Gap Sharing	244

9.1.2.1b	NE-DC: Measurement Gap Sharing	245

9.1.2.1c	NR-DC: Measurement Gap Sharing	246

9.1.3	UE Measurement capability	247

9.1.3.1	EN-DC: Monitoring of multiple layers using gaps	247

9.1.3.1a	SA: Monitoring of multiple layers using gaps	247

9.1.3.1b	NE-DC: Monitoring of multiple layers using gaps	248

9.1.3.1c	NR-DC: Monitoring of multiple layers using gaps	248

9.1.3.2	EN-DC: Maximum allowed layers for multiple monitoring	249

9.1.3.2a	SA: Maximum allowed layers for multiple monitoring	250

9.1.3.2b	NE-DC: Maximum allowed layers for multiple monitoring	250

9.1.3.2c	NR-DC: Maximum allowed layers for multiple monitoring	251

9.1A.3.2	Void	252

9.1.3A	UE Measurement capability under operation mode with CCA	252

9.1.3A.1	EN-DC: Monitoring of multiple layers using gaps under CCA	252

9.1.3A.1A	SA: Monitoring of multiple layers using gaps under CCA	252

9.1.3A.2	EN-DC: Maximum allowed layers for multiple monitoring under CCA	252

9.1A.3.2a	Void	253

9.1.3A.2A	SA: Maximum allowed layers for multiple monitoring under CCA	253

9.1.4	Capabilities for Support of Event Triggering and Reporting Criteria	253

9.1.4.1	Introduction	253

9.1.4.2	Requirements	253

9.1.5	Carrier-specific scaling factor	257

9.1.5.1	Monitoring of multiple layers outside gaps	257

9.1.5.1.1	EN-DC mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	259

9.1.5.1.2	SA mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	260

9.1.5.1.3	NR-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed outside gaps	261

9.1.5.1.4	NE-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based measurements performed outside gaps	262

9.1.5.2	Monitoring of multiple layers within gaps	263

9.1.5.2.1	EN-DC mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	264

9.1.5.2.2	SA mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	266

9.1.5.2.3	NE-DC: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed within gaps	267

9.1.5.2.4	NR-DC: carrier-specific scaling factor for SSB-based and CSI-RS-based L3 measurements performed within gaps	269

9.1.5.2.5	SA mode: carrier-specific scaling factor for PRS-based measurements performed within gaps	271

9.1.5.2.6	NE-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	271

9.1.5.2.7	NR-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	271

9.1.6	Minimum requirement at transitions	271

9.2	NR intra-frequency measurements	271

9.2.1	Introduction	271

9.2.2	Requirements applicability	272

9.2.3	Number of cells and number of SSB	272

9.2.3.1	Requirements for FR1	272

9.2.3.2	Requirements for FR2	272

9.2.4	Measurement Reporting Requirements	273

9.2.4.1	Periodic Reporting	273

9.2.4.2	Event-triggered Periodic Reporting	273

9.2.4.3	Event Triggered Reporting	273

9.2.5	Intrafrequency measurements without measurement gaps	274

9.2.5.1	Intrafrequency cell identification	274

9.2.5.2	Measurement period	276

9.2.5.3	Scheduling availability of UE during intra-frequency measurements	278

9.2.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	278

9.2.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	278

9.2.5.3.3	Scheduling availability of UE performing measurements on FR2	279

9.2.5.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	279

9.2.5.4	SFTD Measurements between PCell and PSCell	280

9.2.5.4.1	Introduction	280

9.2.5.4.2	SFTD Measurement delay	280

9.2.5.4.3	SFTD Measurement Reporting Delay	280

9.2.6	Intra-frequency measurements with measurement gaps	281

9.2.6.1	Void	281

9.2.6.2	Intra-frequency cell identification	281

9.2.6.3	Intrafrequency Measurement Period	282

9.2A	NR intra-frequency measurements with CCA	283

9.2A.1	Introduction	283

9.2A.2	Requirements applicability	284

9.2A.3	Number of cells and number of SSB	284

9.2A.4	Measurement Reporting Requirements	284

9.2A.5	Intra-frequency measurements without measurement gaps	285

9.2A.5.2	Measurement period	287

9.2A.5.3	Scheduling availability of UE during intra-frequency measurements	288

9.2A.5.3.1	Scheduling availability of UE performing measurements in TDD bands	288

9.2A.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH	289

9.2A.6	Intra-frequency measurements with measurement gaps	289

9.2A.6.1	Intra-frequency cell identification	289

9.2A.6.2	Intra-frequency Measurement Period	290

9.2A.7	Intra-frequency RSSI and Channel occupancy measurements	291

9.2A.7.1	Intra-frequency RSSI measurements	291

9.2A.7.2	Intra-frequency Channel occupancy measurements	292

9.2A.7.3	Scheduling restriction during RSSI and Channel Occupancy measurements	293

9.3	NR inter-frequency measurements	293

9.3.1	Introduction	293

9.3.2	Requirements applicability	294

9.3.2.1	Void	294

9.3.2.2	Void	294

9.3.3	Number of cells and number of SSB	294

9.3.3.1	Requirements for FR1	294

9.3.3.2	Requirements for FR2	294

9.3.4	Inter-frequency measurement with measurement gaps	294

9.3.4.1	Void	296

9.3.4.2	Void	296

9.3.5	Inter-frequency measurements	296

9.3.5.1	Void	296

9.3.5.2	Void	296

9.3.5.3	Void	296

9.3.6	Inter-frequency measurements reporting requirements	296

9.3.6.1	Periodic Reporting	296

9.3.6.2	Event-triggered Periodic Reporting	297

9.3.6.3	Event-triggered Reporting	297

9.3.7	Void	297

9.3.8	Inter-frequency SFTD measurement requirements	297

9.3.8.1	Introduction	297

9.3.8.2	SFTD Measurement delay	297

9.3.8.3	SFTD Measurement reporting delay	298

9.3.9	Inter frequency measurements without measurement gaps	299

9.3.9.1	Inter frequency Cell identification	299

9.3.9.2	Measurement period	300

9.3.9.3	Scheduling availability of UE during inter-frequency measurements	301

9.3.9.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	301

9.3.9.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	302

9.3.9.3.3	Scheduling availability of UE performing measurements on FR2	302

9.3.9.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	302

9.3A	NR inter-frequency measurements in carrier frequencies with CCA	303

9.3A.1	Introduction	303

9.3A.2	Requirements applicability	303

9.3A.3	Number of cells and number of SSB	303

9.3A.3.1	Requirements	303

9.3A.4	Inter-frequency cell identification	304

9.3A.5	Inter-frequency measurements	305

9.3A.6	NR Inter-frequency measurements reporting requirements	306

9.3A.6.1	Periodic Reporting	306

9.3A.6.2	Event-triggered Periodic Reporting	306

9.3A.6.3	Event-triggered Reporting	306

9.3A.8	Inter-frequency RSSI measurements	306

9.3A.9	Inter-frequency channel occupancy measurements	307

9.4	Inter-RAT measurements	307

9.4.1	Introduction	307

9.4.2	NR − E-UTRAN FDD measurements	309

9.4.2.1	Introduction	309

9.4.2.2	Requirements when no DRX is used	309

9.4.2.3	Requirements when DRX is used	310

9.4.2.4	Measurement reporting requirements	311

9.4.2.4.1	Periodic Reporting	311

9.4.2.4.2	Event-Triggered Periodic Reporting	311

9.4.2.4.3	Event-Triggered Reporting	311

9.4.3	NR − E-UTRAN TDD measurements	311

9.4.3.1	Introduction	311

9.4.3.2	Requirements when no DRX is used	311

9.4.3.3	Requirements when DRX is used	312

9.4.3.4	Measurement reporting requirements	314

9.4.3.4.1	Periodic Reporting	314

9.4.3.4.2	Event-Triggered Periodic Reporting	314

9.4.3.4.3	Event-Triggered Reporting	314

9.4.4	Inter-RAT RSTD measurements	314

9.4.4.1	NR − E-UTRAN FDD RSTD measurements	314

9.4.4.1.1	Introduction	314

9.4.4.1.2	Requirements	315

9.4.4.2	NR − E-UTRAN TDD RSTD measurements	318

9.4.4.2.1	Introduction	318

9.4.4.2.2	Requirements	319

9.4.5	Inter-RAT E-CID measurements	322

9.4.5.1	NR−E-UTRAN FDD E-CID RSRP and RSRQ measurements	322

9.4.5.1.1	Introduction	322

9.4.5.1.2	Requirements	322

9.4.5.1.3	Measurement Reporting Delay	322

9.4.5.2	NR−E-UTRAN TDD E-CID RSRP and RSRQ measurements	322

9.4.5.2.1	Introduction	322

9.4.5.2.2	Requirements	322

9.4.5.2.3	Measurement Reporting Delay	323

9.4.6	NR − UTRAN FDD measurements	323

9.4.6.1	Introduction	323

9.4.6.2	Requirements when no DRX is used	323

9.4.6.3	Requirements when DRX is used	324

9.4.7	NR – E-UTRAN measurements with autonomous gaps	326

9.4.7.1	CGI identification of an E-UTRA cell with autonomous gaps	326

The requirement for identifying a new CGI of an E-UTRA cell within Tidentify_CGI, E-UTRAN is applicable when no DRX is used as well as when any of the DRX cycles specified in TS 38.331 [2] is used.9.4.7.2	CGI reporting delay	326

9.5	L1-RSRP measurements for Reporting	327

9.5.1	Introduction	327

9.5.2	Requirements applicability	327

9.5.3	Measurement Reporting Requirements	327

9.5.3.1	Periodic Reporting	328

9.5.3.2	Semi-Persistent Reporting	328

9.5.3.3	Aperiodic Reporting	328

9.5.4	L1-RSRP measurement requirements	328

9.5.4.1	SSB based L1-RSRP Reporting	328

9.5.4.2	CSI-RS based L1-RSRP Reporting	330

9.5.4A	Void	333

9.5.4A.1	Void	333

9.5.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	333

9.5.5.1	Measurement restriction for SSB based L1-RSRP	333

9.5.5.2	Measurement restriction for CSI-RS based L1-RSRP	334

9.5.6	Scheduling availability of UE during L1-RSRP measurement	334

9.5.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	334

9.5.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	334

9.5.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	335

9.5.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	336

9.5A	L1-RSRP measurements for Reporting under CCA	336

9.5A.1	Introduction	336

9.5A.2	Requirements applicability	336

9.5A.3	Measurement Reporting Requirements	336

9.5A.3.1	Periodic Reporting	337

9.5A.3.2	Semi-Persistent Reporting	337

9.5A.3.3	Aperiodic Reporting	337

9.5A.4	L1-RSRP measurement requirements	337

9.5A.4.1	SSB based L1-RSRP Reporting	337

9.5A.5	Measurement restriction for L1-RSRP measurement	338

9.5A.5.1	Measurement restriction for SSB based L1-RSRP	338

9.5A.6	Scheduling availability of UE during L1-RSRP measurement	339

9.5A.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH	339

9.5A.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH	339

9.5A.6.3	Scheduling availability of UE performing L1-RSRP measurement in case of FR1-FR2 inter-band CA	339

9.6	NE-DC: Measurements	339

9.6.1	Introduction	339

9.6.2	SFTD Measurements	339

9.6.2.1	Introduction	339

9.6.2.2	SFTD Measurement requirements	339

9.7	Cross Link Interference measurements	340

9.7.1	Introduction	340

9.7.2	SRS-RSRP measurements	340

9.7.2.1	Introduction	340

9.7.2.2	Requirements applicability	341

9.7.2.3	Measurement Reporting Requirements	341

9.7.2.3.1	Periodic Reporting	341

9.7.2.3.2	Event-triggered Periodic Reporting	341

9.7.2.3.3	Event Triggered Reporting	341

9.7.2.4	Measurement capability	341

9.7.2.5	SRS-RSRP measurement period	341

9.7.3	CLI-RSSI measurements	342

9.7.3.1	Introduction	342

9.7.3.2	Requirements applicability	342

9.7.3.3	Measurement Reporting Requirements	342

9.7.3.3.1	Periodic Reporting	342

9.7.3.3.2	Event-triggered Periodic Reporting	342

9.7.3.3.3	Event Triggered Reporting	342

9.7.3.4	Measurement capability	343

9.7.3.5	CLI-RSSI measurement period	343

9.7.4	Scheduling availability of UE during CLI measurements	343

9.7.4.1	Scheduling availability of UE performing measurement on FR1	343

9.7.4.2	Scheduling availability of UE performing measurement on FR2	344

9.8	L1-SINR measurements for Reporting	344

9.8.1	Introduction	344

9.8.2	Requirements applicability	345

9.8.3	Measurement Reporting Requirements	345

9.8.3.1	Periodic Reporting	345

9.8.3.2	Semi-Persistent Reporting	346

9.8.4	L1-SINR measurement requirements	346

9.8.4.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	346

9.8.4.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	349

9.8.5	Measurement restriction for L1-SINR measurement	351

9.8.5.1	Measurement restriction if SSB configured for L1-SINR Measurement	351

9.8.5.2	Measurement restriction if CSI-RS configured for L1-SINR measurement	352

9.8.5.3	Measurement restriction if CSI-IM configured for L1-SINR measurement	352

9.8.6	Scheduling availability of UE during L1-SINR measurement	353

9.8.6.1	Scheduling availability of UE performing L1-SINR measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	353

9.8.6.2	Scheduling availability of UE performing L1-SINR measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	353

9.8.6.4	Scheduling availability of UE performing L1-SINR measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	354

9.9	 NR measurements for positioning	354

9.9.1	Introduction	354

9.9.2	RSTD measurements	355

9.9.2.1	Introduction	355

9.9.2.2	Requirements Applicability	355

9.9.2.3	Measurement Capability	355

9.9.2.4	Measurement Reporting Requirements	355

9.9.2.4.1	Void	355

9.9.2.4.2	Void	355

9.9.2.4.3	Void	355

9.9.2.5	Measurements Period Requirements	355

9.9.2.6	Void	357

9.9.3	PRS-RSRP measurements	357

9.9.3.1	Introduction	357

9.9.3.2	Requirements applicability	357

9.9.3.3	Measurement Capability	357

9.9.3.4	Measurement Reporting Requirements	358

9.9.3.5	Measurement Period Requirements	358

9.9.4	UE Rx-Tx time difference measurements	360

9.9.4.1 Introduction	360

9.9.4.2 Requirements Applicability	360

9.9.4.3 Measurement Capability	360

9.9.4.4 Measurement Reporting Requirements	360

9.9.4.5 Measurement Period Requirements	360

9.9.5.1	Introduction	362

9.9.5.2	Measurement Requirements	362

9.9.5.2.1	Intra-frequency Measurement Requirements	362

9.9.5.2.2	Inter-frequency Measurement Requirements	363

9.9.5.2.3	Measurement Reporting Delay	363

9.10	CSI-RS based L3 measurements	363

9.10.1	Introduction	363

9.10.2	CSI-RS based intra-frequency measurements	364

9.10.2.1	Introduction	364

9.10.2.2	Requirements applicability	364

9.10.2.3	Number of cells and number of CSI-RS	365

9.10.2.3.1	Requirements for FR1	365

9.10.2.3.2	Requirements for FR2	365

9.10.2.4	Measurement Reporting Requirements	366

9.10.2.4.1	Periodic Reporting	366

9.10.2.4.2	Event-triggered Periodic Reporting	366

9.10.2.4.3	Event Triggered Reporting	366

9.10.2.5	Intra-frequency measurements without measurement gaps	366

9.10.2.6	Scheduling availability of UE during CSI-RS based intra-frequency measurements	367

9.10.2.6.1	Scheduling availability of UE performing CSI-RS based measurements in TDD bands	368

9.10.2.6.2	Scheduling availability of UE performing CSI-RS based measurements in FR2	368

9.10.3	CSI-RS based Inter-frequency measurements	368

9.10.3.1	Introduction	368

9.10.3.2	Requirements applicability	368

9.10.3.3	Number of cells and number of CSI-RS resources	369

9.10.3.3.1	Requirements for FR1	369

9.10.3.3.2	Requirements for FR2	369

9.10.3.4	Measurements reporting requirements	369

9.10.3.4.1	Periodic Reporting	369

9.10.3.4.2	Event-triggered Periodic Reporting	369

9.10.3.4.3	Event-triggered Reporting	370

9.10.3.5	Inter frequency measurements with measurement gaps	370

9.11	NR measurements with autonomous gaps	371

9.11.1	Introduction	371

9.11.2	CGI identification of an NR cell with autonomous gaps	371

9.11.3	CGI reporting delay	372

10	Measurement Performance requirements	372

10.1	NR measurements	372

10.1.1	Introduction	372

10.1.2	Intra-frequency RSRP accuracy requirements for FR1	373

10.1.2.1	Intra-frequency SS-RSRP accuracy requirements	373

10.1.2.1.1	Absolute SS-RSRP Accuracy	373

10.1.2.1.2	Relative SS-RSRP Accuracy	374

10.1.2.2	Void	375

10.1.2.3	Intra-frequency CSI-RSRP accuracy requirements	375

10.1.2.3.1	Absolute CSI-RSRP Accuracy	375

10.1.2.3.2	Relative CSI-RSRP Accuracy	376

10.1.2B	Intra-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	377

10.1.2B.1	Intra-frequency SS-RSRP accuracy requirements	377

10.1.2B.1.1	Absolute SS-RSRP Accuracy	377

10.1.3	Intra-frequency RSRP accuracy requirements for FR2	378

10.1.3.1	Intra-frequency SS-RSRP accuracy requirements	378

10.1.3.1.1	Absolute SS-RSRP Accuracy	378

10.1.3.1.2	Relative SS-RSRP Accuracy	379

10.1.3.2	Void	380

10.1.3.3	Intra-frequency CSI-RSRP accuracy requirements	380

10.1.3.3.1	Absolute CSI-RSRP Accuracy	380

10.1.3.3.2	Relative CSI-RSRP Accuracy	380

10.1.3B	Intra-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	381

10.1.3B.1	Intra-frequency SS-RSRP accuracy requirements	381

10.1.3B.1.1	Absolute SS-RSRP Accuracy	381

10.1.4	Inter-frequency RSRP accuracy requirements for FR1	382

10.1.4.1	Inter-frequency SS-RSRP accuracy requirements	382

10.1.4.1.1	Absolute Accuracy of SS-RSRP in FR1	382

10.1.4.1.2	Relative Accuracy of SS-RSRP in FR1	383

10.1.4.2	Void	384

10.1.4.3	Inter-frequency CSI-RSRP accuracy requirements	384

10.1.4.3.1	Absolute Accuracy of CSI-RSRP in FR1	384

10.1.4.3.2	Relative Accuracy of CS-RSRP in FR1	385

10.1.4B	Inter-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	386

10.1.4B.1	Inter-frequency SS-RSRP accuracy requirements	386

10.1.4B.1.1	Absolute Accuracy of SS-RSRP in FR1	386

10.1.5	Inter-frequency RSRP accuracy requirements for FR2	387

10.1.5.1	Inter-frequency SS-RSRP accuracy requirements	387

10.1.5.1.1	Absolute SS-RSRP Accuracy	387

10.1.5.1.2	Relative SS-RSRP Accuracy	388

10.1.5.2	Void	389

10.1.5.3	Inter-frequency CSI-RSRP accuracy requirements	389

10.1.5.3.1	Absolute CSI-RSRP Accuracy	389

10.1.5.3.2	Relative CSI-RSRP Accuracy	389

10.1.5B	Inter-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	390

10.1.5B.1	Inter-frequency SS-RSRP accuracy requirements	390

10.1.5B.1.1	Absolute SS-RSRP Accuracy	390

10.1.6 RSRP Measurement Report Mapping	391

10.1.7	Intra-frequency RSRQ accuracy requirements for FR1	393

10.1.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	393

10.1.7.1.1	Absolute SS-RSRQ Accuracy in FR1	393

10.1.7.2	Intra-frequency CSI-RSRQ accuracy requirements	394

10.1.7.2.1	Absolute CSI-RSRQ Accuracy	394

10.1.7B	Intra-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	395

10.1.7B.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	395

10.1.7B.1.1	Absolute SS-RSRQ Accuracy in FR1	395

10.1.8	Intra-frequency RSRQ accuracy requirements for FR2	396

10.1.8.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	396

10.1.8.1.1	Absolute SS-RSRQ Accuracy in FR2	396

10.1.8.2	Intra-frequency CSI-RSRQ accuracy requirements	397

10.1.8.2.1	Absolute CSI-RSRQ Accuracy	397

10.1.8B	Intra-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	398

10.1.8B.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	398

10.1.8B.1.1	Absolute SS-RSRQ Accuracy in FR2	398

10.1.9	Inter-frequency RSRQ accuracy requirements for FR1	399

10.1.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	399

10.1.9.1.1	Absolute Accuracy of SS-RSRQ in FR1	399

10.1.9.1.2	Relative Accuracy of SS-RSRQ in FR1	400

10.1.9.2	Inter-frequency CSI-RSRQ accuracy requirements	401

10.1.9.2.1	Absolute CSI-RSRQ Accuracy	401

10.1.9.2.2	Relative CSI-RSRQ Accuracy	402

10.1.9B	Inter-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	403

10.1.9B.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	403

10.1.9B.1.1	Absolute Accuracy of SS-RSRQ in FR1	403

10.1.10	Inter-frequency RSRQ accuracy requirements for FR2	404

10.1.10.2	Inter-frequency CSI-RSRQ accuracy requirements	406

10.1.10.2.1	Absolute CSI-RSRQ Accuracy	406

10.1.10.2.2	Relative CSI-RSRQ Accuracy	406

10.1.10B	Inter-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	407

10.1.10B.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	407

10.1.10B.1.1	Absolute Accuracy of SS-RSRQ in FR2	407

10.1.11	RSRQ report mapping	408

10.1.12	Intra-frequency SINR accuracy requirements for FR1	408

10.1.12.2	Intra-frequency CSI-SINR accuracy requirements in FR1	409

10.1.12.2.1	Absolute CSI-SINR Accuracy in FR1	409

10.1.13	Intra-frequency SINR accuracy requirements for FR2	410

10.1.13.2	Intra-frequency CSI-SINR accuracy requirements in FR2	411

10.1.13.2.1	Absolute CSI-SINR Accuracy in FR2	411

10.1.14	Inter-frequency SINR accuracy requirements for FR1	412

10.1.14.2	Inter-frequency CSI-SINR accuracy requirements in FR1	414

10.1.14.2.1	Aboslute Accuracy of CSI-SINR in FR1	414

10.1.15	Inter-frequency SINR accuracy requirements for FR2	416

10.1.15.2	Inter-frequency CSI-SINR accuracy requirements in FR2	418

10.1.15.2.1	Aboslute Accuracy of CSI-SINR in FR2	418

10.1.15.2.2	Relative Accuracy of CSI-SINR in FR2	418

10.1.16	SINR report mapping	419

10.1.17	Power Headroom	420

10.1.18	PCMAX,c,f	421

10.1.19	L1-RSRP accuracy requirements for FR1	421

10.1.20	L1-RSRP accuracy requirements for FR2	425

10.1.21	SFTD accuracy requirements	428

10.1.22	CLI measurement accuracy requirements	432

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	436

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	436

10.1.23	RSTD Measurements	438

10.1.23.1	Introduction	438

10.1.23.2	Measurement Accuracy Requirements	439

10.1.23.3	Report mapping	443

10.1.23.3.1	 Absolute DL RSTD Measurement Reporting	443

10.1.23.3.2	 Differential Reporting for DL RSTD Measurement	445

10.1.23.3.3	Additional Path Report Mapping for DL RSTD	447

10.1.24	PRS-RSRP Measurements	449

10.1.24.1	Introduction	449

10.1.24.2	Measurement Accuracy Requirements	449

10.1.24.2.1 Absolute PRS RSRP accuracy	449

10.1.24.2.2 Relative PRS RSRP accuracy	451

10.1.24.3	Report mapping	453

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	453

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	454

10.1.25	UE Rx-Tx Time Difference Measurements	456

10.1.25.1	Introduction	456

10.1.25.2	Measurement Accuracy Requirements	457

10.1.25.3	Report mapping	457

10.1.25.3.1	 Absolute UE Rx-Tx Measurement Report Mapping	457

10.1.25.3.2	 Differential UE Rx-Tx Measurement Report Mapping	459

10.1.25.3.3	 Additional Path Report Mapping for UE Rx-Tx Time Difference	460

10.1.26	FR2 P-MPR report	462

10.1.26.1	Report mapping	462

10.1.27	L1-SINR accuracy requirements for FR1	462

10.1.28	L1-SINR accuracy requirements for FR2	470

10.1.29	Intra-frequency RSRQ accuracy requirements under CCA	477

10.1.29.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	477

10.1.29.1.1	Absolute SS-RSRQ Accuracy	477

10.1.30	Inter-frequency RSRQ accuracy requirements under CCA	477

10.1.30.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	477

10.1.30.1.1	Aboslute Accuracy of SS-RSRQ	477

10.1.30.1.2	Relative Accuracy of SS-RSRQ	478

10.1.31	Intra-frequency SINR accuracy requirements under CCA	479

10.1.31.1	Intra-frequency SS-SINR accuracy requirements in FR1	479

10.1.31.1.1	Absolute SS-SINR Accuracy	479

10.1.32	Inter-frequency SINR accuracy requirements under CCA	479

10.1.32.1	Inter-frequency SS-SINR accuracy requirements in FR1	479

10.1.32.1.1	Aboslute Accuracy of SS-SINR	479

10.1.32.1.2	Relative Accuracy of SS-SINR	480

10.1.33	L1-RSRP accuracy requirements under CCA	481

10.1.33.1	SSB based L1-RSRP accuracy requirements in FR1	481

10.1.33.1.1	Absolute Accuracy	481

10.1.33.1.2	Relative Accuracy	481

10.1.34	RSSI measurements under CCA	482

10.1.34.1	Intra-frequency absolute RSSI measurement accuracy requirements in FR1	482

10.1.34.2	Inter-frequency absolute RSSI measurement accuracy requirements in FR1	482

10.1.34.3	RSSI measurement report mapping	482

10.1.35	Channel occupancy measurements under CCA	483

10.1.35.1	Intra-frequency channel occupancy measurement accuracy requirements in FR1	483

10.1.35.2	Inter-frequency channel occupancy measurement accuracy requirements in FR1	483

10.1.36	Intra-frequency RSRP accuracy requirements under CCA	483

10.1.36.1	Intra-frequency SS-RSRP accuracy requirements in FR1	483

10.1.36.1.1	Absolute SS-RSRP Accuracy	483

10.1.36.1.2	Relative SS-RSRP Accuracy	484

10.1.37	Inter-frequency RSRP accuracy requirements under CCA	485

10.1.37.1	Inter-frequency SS-RSRP accuracy requirements in FR1	485

10.1.37.1.1	Absolute Accuracy of SS-RSRP	485

10.1.37.1.2	Relative Accuracy of SS-RSRP	485

10.2	E-UTRAN measurements	486

10.2.1	Introduction	486

10.2.2	E-UTRAN RSRP measurements	486

10.2.3	E-UTRAN RSRQ measurements	486

10.2.4	E-UTRAN RSTD measurements	487

10.2.5	E-UTRAN RS-SINR measurements	487

10.2.6	E-UTRAN RSRP measurements for CA/DC Idle Mode Measurements	487

10.2.7	E-UTRAN RSRQ measurements for CA/DC Idle Mode Measurements	488

10.3	UTRAN FDD Measurements	488

10.3.1	UTRAN FDD CPICH RSCP	488

10.3.2	UTRAN FDD CPICH Ec/No	489

10.4	V2X measurements	489

10.4.1	Introduction	489

10.4.2	Intra-frequency PSBCH-RSRP accuracy requirements for FR1	489

10.4.2.1	PSBCH-RSRP Absolute Accuracy	489

10.4.2.2	PSBCH-RSRP Relative Accuracy	490

10.4.3	Intra-Frequency SL-RSSI Measurement Accuracy Requirements for FR1	491

10.4.3.1	Absolute SL-RSSI Accuracy	491

10.4.4	Intra-Frequency L1 SL-RSRP Measurement Accuracy Requirements for FR1	491

10.4.4.1	Absolute L1 SL-RSRP Accuracy	491

11	Void	492

12	V2X Requirements	492

12.1	Introduction	492

12.2	UE Transmit Timing	492

12.2.1	Introduction	492

12.2.2	GNSS as synchronization reference source	493

12.2.3	NR Cell as synchronization reference source	493

12.2.4	E-URTAN Cell as synchronization reference source	493

12.2.5	SyncRef UE as synchronization reference source	494

12.3	Initiation/Cease of SLSS Transmissions	494

12.3.1	Introduction	494

12.3.1.1	Initiation/Cease of SLSS transmissions with NR cell as synchronization reference source	494

12.3.1.2	Initiation/Cease of SLSS transmissions with EUTRAN cell as synchronization reference source	495

12.3.1.3	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	496

12.3.1.4	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	496

12.4	Selection / Reselection of V2X Synchronization Reference Source	497

12.5	L1 SL-RSRP measurements	498

12.5.1	Introduction	498

12.5.2	SL-RSRP measurements	498

12.6	Congestion Control measurements	498

12.7	Interruption	498

12.7.1	Interruptions to WAN due to V2X Sidelink Communication	498

12.7.2	V2X Sidelink Communication Dropping due to synchronization source change	499

12.7.3	Interruptions to WAN due to switching between E-UTRA V2X Sidelink and NR V2X Sidelink	501

12.8	Reliability of GNSS signal	501

12.9	Scheduling availability	501

12.9.1	Scheduling availability of UE switching between E-UTRA sidelink and NR sidelink	501

13	Measurement Performance Requirements for NR gNB	502

13.1	UL-RTOA	502

13.1.1	Report mapping	502

13.2	gNB Rx-Tx time difference	504

13.2.1	Report mapping	504

13.2.2	Measurement Accuracy Requirements	506

13.2.2.1	Introduction	506

13.3	UL SRS RSRP measurement	507

13.3.1	Report mapping	507

13.3.2	Measurement accuracy requirements	508

13.3.2.1	Introduction	508

13.3.2.2	Requirements	508

13.4	AoA/ZoA	509

13.4.1	Report mapping	509

Annex A (normative): Test Cases	423

A.1	Purpose of annex	423

A.2	Requirement classification for statistical testing	423

A.2.1	Types of requirements in TS 38.133	423

A.2.1.1	Time and delay requirements on UE higher layer actions	423

A.2.1.2	Measurements of power levels, relative powers and time	423

A.2.1.3	Implementation requirements	424

A.2.1.4	Physical layer timing requirements	424

A.2.1.5	Requirements under CCA	424

A.3	RRM test configurations	425

A.3.1	Reference measurement channels	425

A.3.1.1	PDSCH	425

A.3.1.1.1	FDD	425

A.3.1.1.2	TDD	426

A.3.1.2	CORESET for RMSI scheduling	429

A.3.1.2.1	FDD	429

A.3.1.2.2	TDD	430

A.3.1.3	CORESET for RMC scheduling	433

A.3.1.3.1	FDD	433

A.3.1.3.2	TDD	434

A.3.1.4	TDD UL/DL configuration	437

A.3.1A	Reference measurement channels under CCA	439

A.3.1A.1	PDSCH	439

A.3.1A.1.1	TDD	439

A.3.1A.2	CORESET for RMSI scheduling	440

A.3.1A.2.1	TDD	440

A.3.1A.3	CORESET for RMC scheduling	441

A.3.1A.3.1	TDD	441

A.3.1A.4	TDD UL/DL configuration	442

A.3.1A.5	RMC burst transmission model	442

RMC burst transmission is scheduled outside discovery burst transmission window. If transmission occurred in the previous slot, transmission is muted for a duration of one slot. Additionaly, if the start time of the candidate RMC burst transmission is within 5 slots of the start of the discovery burst transmission window, RMC transmission is not performed.A.3.2	OFDMA channel noise generator (OCNG)	442

A.3.2.1	Generic OFDMA Channel Noise Generator (OCNG)	442

A.3.2.1.1	OCNG pattern 1: Generic OCNG pattern for all unused REs	443

A.3.2.1.2	OCNG pattern 2: Generic OCNG pattern for all unused REs for 2AoA setup	443

A.3.2.1.3	OCNG pattern 3: Generic OCNG pattern for unused REs in the same bandwidth as CORESET	444

A.3.2.1.4	OCNG pattern 4: Generic OCNG pattern for all unused REs outside SSB slot(s)	444

A.3.2.2	Void	445

A.3.3	Reference DRX configurations	445

A.3.3.1	DRX Configuration 1: DRX cycle = 40 ms and TAT = 500 ms	445

A.3.3.2	DRX Configuration 2: DRX cycle = 640 ms and TAT = 500 ms	446

A.3.3.3	DRX Configuration 3: DRX cycle = 40 ms and TAT = Infinity	446

A.3.3.4	DRX Configuration 4: DRX cycle = 160 ms and TAT = Infinity	446

A.3.3.5	DRX Configuration 5: DRX cycle = 320 ms and TAT = Infinity	447

A.3.3.6	DRX Configuration 6: DRX cycle = 320 ms and TAT = 500 ms	447

A.3.3.7	DRX Configuration 7: DRX cycle = 640 ms and TAT = Infinity	447

A.3.3.8	DRX Configuration 8: DRX cycle = 320 ms and TAT = Infinity	448

A.3.3.9	DRX Configuration 9: DRX cycle = 40 ms and TAT = 500 ms	448

A.3.3.10	DRX Configuration 10: DRX cycle = 640 ms and TAT = 500 ms	448

A.3.3.11	DRX Configuration 11: DRX cycle = 20 ms and TAT = Infinity	449

A.3.3.12	DRX Configuration 12: DRX cycle = 640 ms and TAT = Infinity	449

A.3.4	Test Cases with Different Channel Bandwidths	449

A.3.4.1	Test Cases with Different E-UTRA Channel Bandwidths	449

A.3.4.1.1	Introduction	449

A.3.4.1.2	Principle of testing	449

A.3.5	Test Cases for Synchronous and Asynchronous DC Operations	450

A.3.5.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations	450

A.3.5.1.1	Introduction	450

A.3.5.1.2	Principle of Testing	450

A.3.6	Antenna configurations	450

A.3.6.1	Antenna configurations for FR1	450

A.3.6.1.1	Antenna connection for 4 Rx capable UEs	450

A.3.6.1.1.1	Introduction	450

A.3.6.1.1.2	Principle of testing	450

A.3.6.2	Antenna configurations for FR2	453

A.3.6A	Antenna configurations with unlicensed bands	453

A.3.6A.1	Antenna configurations for FR1	453

A.3.6A.1.1	Antenna connection for 4 Rx capable UEs	453

A.3.6A.1.1.1	Introduction	453

A.3.6A.1.1.2	Principle of testing	453

A.3.7	EN-DC test setup	455

A.3.7.1	Introduction	455

A.3.7.2	E-UTRAN Serving Cell Parameters	455

A.3.7.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	455

A.3.7.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	456

A.3.7A	NR FR1-FR2 test setup	457

A.3.7B	EN-DC test setup with unlicensed bands	458

A.3.7B.1	Introduction	458

A.3.7B.2	E-UTRAN Serving Cell Parameters	458

A.3.7B.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) under CCA in FR1	458

A.3.8	PRACH configurations	459

A.3.8.1	Introduction	459

A.3.8.2	PRACH configurations in FR1	459

A.3.8.2.1	FR1 PRACH configuration 1	459

A.3.8.2.2	FR1 PRACH configuration 2	460

A.3.8.2.3	FR1 PRACH configuration 3	461

A.3.8.2.4	FR1 PRACH configuration 4	462

A.3.8.3	PRACH configurations in FR2	463

A.3.8.3.1	FR2 PRACH configuration 1	463

A.3.8.3.2	FR2 PRACH configuration 2	464

A.3.8.3.3	FR2 PRACH configuration 3	465

A.3.8.3.4	FR2 PRACH configuration 4	466

A.3.8A	PRACH configurations under CCA	467

A.3.8A.1	Introduction	467

A.3.8A.2	PRACH configurations in FR1	467

A.3.8A.2.1	FR1 PRACH configuration 1 under CCA	467

A.3.8A.2.2	FR1 PRACH configuration 2 under CCA	468

A.3.9	BWP configurations	469

A.3.9.1	Introduction	469

A.3.9.2	Downlink BWP configurations	470

A.3.9.2.1	Initial BWP	470

A.3.9.2.2	Dedicated BWP	470

A.3.9.3	Uplink BWP configurations	470

A.3.9.3.1	Initial BWP	470

A.3.9.3.2	Dedicated BWP	471

A.3.10	SSB Configurations	471

A.3.10.1	SSB Configurations for FR1	471

A.3.10.1.1	SSB pattern 1 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	471

A.3.10.1.5	SSB pattern 5 in FR1: SSB allocation for SSB SCS=15 kHz starting from odd SFN in 10 MHz	473

A.3.10A	SSB Configurations under CCA	478

A.3.10A.1	SSB Configurations under CCA for FR1	478

A.3.10A.1.1	SSB pattern 1 under CCA for semi-static channel access: SSB allocation for SSB SCS=30kHz in 40MHz	478

A.3.10A.1.2	SSB pattern 2 under CCA for dynamic channel access: SSB allocation for SSB SCS=30kHz in 40MHz	479

A.3.10A.1.3	SSB pattern 3 under CCA for semi-static channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	479

A.3.10A.1.4	SSB pattern 4 under CCA for dynamic channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	480

A.3.11	SMTC Configurations	480

A.3.11.1	SMTC pattern 1: SMTC period = 20 ms with SMTC duration = 1 ms	480

A.3.11.2	SMTC pattern 2: SMTC period = 20 ms with SMTC duration = 5 ms	480

A.3.11.3	SMTC pattern 3: SMTC period = 160 ms with SMTC duration = 1 ms	481

A.3.11.4	SMTC pattern 4: SMTC period = 20 ms with SMTC duration = 1 ms	481

A.3.11.5	SMTC pattern 5: SMTC period = 20 ms with SMTC duration = 5 ms	481

A.3.12	Test Cases with Different CC Configurations	481

A.3.12.1 EN-DC Test Cases with Different EN-DC Configurations	481

A.3.12.1.1	Introduction	481

A.3.12.1.2	Principle of testing	481

A.3.12.2	Carrier Aggregation Test Cases with Different CA Configurations	482

A.3.12.2.1	Introduction	482

A.3.12.2.2	Principle of testing	482

A.3.13	Test Cases in SA and EN-DC Operations	482

A.3.13.1	Introduction	482

A.3.13.2	Principle of Testing	482

A.3.14	CSI-RS configurations	482

A.3.14.1	FDD	482

A.3.14.2	TDD	486

A.3.15	Angle of Arrival (AoA) for FR2 RRM test cases	492

A.3.15.1	Setup 1: Single AoA in Rx beam peak direction	492

A.3.15.2	Setup 2: Single AoA in non Rx beam peak direction	492

A.3.15.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction	492

A.3.15.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction	493

A.3.15.3	Setup 3: 2 AoAs	493

A.3.15.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak	493

A.3.15.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction	493

A.3.15.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction	493

A.3.16	TCI State Configuration	494

A.3.16.1	Introduction	494

A.3.16.2	TCI states	494

A.3.17	Configurations of CSI-RS for tracking	494

A.3.17.1	Configuration of CSI-RS for tracking for FR1	494

A.3.17.1.1	FDD	494

A.3.17.1.2	TDD	495

A.3.17.2	Configuration of CSI-RS for tracking for FR2	496

A.3.17.2.1	TDD	496

A.3.18	Additional definitions related to OTA testing for FR2 RRM test cases	497

A.3.18.1	Introduction	497

A.3.18.2	PRACH Power Measurement	497

A.3.19	Test applicability for DAPS handover	497

A.3.19.1	Introduction	497

A.3.19.2	Principle of testing	497

A.3.20	MsgA configurations	498

A.3.20.1	Introduction	498

A.3.20.2	MsgA configurations in FR1	498

A.3.20.2.1	FR1 MsgA configuration 1	498

A.3.20.2.2	FR1 MsgA configuration 2	499

A.3.20.3	MsgA configurations in FR2	501

A.3.20.3.1	FR2 MsgA configuration 1	501

A.3.20.3.2	FR2 MsgA configuration 2	502

A.3.20A	MsgA configurations under CCA	504

A.3.20A.1	Introduction	504

A.3.20A.2	MsgA configurations in FR1	504

A.3.20A.2.1	FR1 MsgA configuration 1 under CCA	504

A.3.20A.2.2	FR1 MsgA configuration 2 under CCA	505

A.3.21	V2X sidelink communication	508

A.3.21.1	Introduction	508

A.3.21.2	Reference resource pool configurations for V2X Sidelink Communication	508

A.3.21.3	Reference measurement channels for V2X Sidelink Communication	514

A.3.22	CSI-IM configurations	514

A.3.22.1	FDD	514

A.3.22.2	TDD	515

A.3.23	Spatial Relation Configuration	517

A.3.23.1	Introduction	517

A.3.23.2	Spatial Relation	517

A.3.24	SRS configuration	518

A.3.25	Channel bandwidth (CBW) configurations	520

A.3.25.1	DL UE specific CBW	520

A.3.25.2	UL UE specific CBW	520

A.3.26	CCA model	520

A.3.26.1	Introduction	520

A.3.26.2	CCA model for operation on a carrier frequency with CCA in FR1	520

A.3.26.2.1	DL CCA model	520

A.3.26.2.2	UL CCA model	521

A.3.27	Test Cases with at Least One Cell on a Carrier Frequency with CCA	522

A.3.27.1	Introduction	522

A.3.27.2	 NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	522

A.3.27.3	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	522

A.3.27.4	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	522

A.3.27.5	E-UTRA Standalone Tests with at Least One NR Cell under CCA	522

A.3.28	Discovery Burst Transmission Window configuration under CCA	523

A.3.28.1	DBT Window pattern 1: DBT Window period = 20 ms with DBT Window duration = 1 ms	523

A.3.29	Testing principles for UE capable of only NR bands with shared spectrum access	523

A.3.29.1	Introduction	523

A.3.29.2	Principle of testing for UE capable of EN-DC with only NR bands with shared spectrum access	523

A.3.29.3	Principle of testing for UE capable of SA operation with only NR bands with shared spectrum access	523

A.3.30	CSI-RS configurations for RRM	524

A.3.30.1	FDD	524

A.3.30.2	TDD	525

A.3.31	PRS Configurations	527

A.3.31.1.	PRS Configurations for FR1	527

A.3.31.1.1.	PRS pattern 1 in FR1: SCS=15 KHz in 10 MHz	527

A.3.31.1.2.	PRS pattern 2 in FR1: SCS=30 KHz in 40 MHz	528

A.3.31.2.	PRS Configurations for FR2	528

A.3.31.2.1.	PRS pattern 1 in FR2: SCS=120 KHz in 100 MHz	528

A.4	EN-DC tests with all NR cells in FR1	529

A.4.1	Void	529

A.4.2	Void	529

A.4.3	RRC_CONNECTED state mobility	529

A.4.3.1	Void	529

A.4.3.2	RRC Connection Mobility Control	529

A.4.3.2.1	Void	529

A.4.3.2.2	Random Access	529

A.4.3.2.2.1	4-step RA type contention based random access test in FR1 for PSCell in EN-DC	529

A.4.3.2.2.2	4-step RA type n on-contention based random access test in FR1 for PSCell in EN-DC	532

A.4.3.2.2.3	2-step RA type contention based random access test in FR1 for PSCell in EN-DC	536

A.4.3.2.2.4	2-step RA type n on-contention based random access test in FR1 for PSCell in EN-DC	539

A.4.3.2.3	Void	543

A.4.4	Timing	543

A.4.4.1	UE transmit timing	543

A.4.4.1.1	NR UE Transmit Timing Test for FR1	543

A.4.4.1.1.1	Test Purpose and environment	543

A.4.4.1.1.2	Test requirements	547

A.4.4.2	UE timer accuracy	547

A.4.4.3	Timing advance	547

A.4.4.3.1	EN-DC FR1 timing advance adjustment accuracy	547

A.4.4.3.1.1	Test Purpose and Environment	547

A.4.4.3.1.2	Test Parameters	547

A.4.4.3.1.3	Test Requirements	551

A.4.5	Signaling characteristics	551

A.4.5.1	Radio link Monitoring	551

A.4.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	552

A.4.5.1.1.1	Test Purpose and Environment	552

A.4.5.1.1.2	Test Requirements	556

A.4.5.1.2	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	556

A.4.5.1.2.1	Test Purpose and Environment	556

A.4.5.1.2.2	Test Requirements	562

A.4.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	562

A.4.5.1.3.1	Test Purpose and Environment	562

A.4.5.1.3.2	Test Requirements	568

A.4.5.1.4	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	568

A.4.5.1.4.1	Test Purpose and Environment	568

A.4.5.1.4.2	Test Requirements	574

A.4.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	574

A.4.5.1.5.1	Test Purpose and Environment	574

A.4.5.1.5.2	Test Requirements	580

A.4.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	580

A.4.5.1.6.1	Test Purpose and Environment	580

A.4.5.1.6.2	Test Requirements	586

A.4.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	586

A.4.5.1.7.1	Test Purpose and Environment	586

A.4.5.1.7.2	Test Requirements	591

A.4.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	592

A.4.5.1.8.1	Test Purpose and Environment	592

A.4.5.1.8.2	Test Requirements	597

A.4.5.2	Interruption	598

A.4.5.2.1	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	598

A.4.5.2.1.1	Test Purpose and Environment	598

A.4.5.2.1.2	Test Requirements	601

A.4.5.2.2	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	601

A.4.5.2.2.1	Test Purpose and Environment	601

A.4.5.2.2.2	Test Requirements	605

A.4.5.2.3	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	605

A.4.5.2.3.1	Test Purpose and Environment	605

A.4.5.2.3.2	Test Requirements	609

A.4.5.2.4	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	610

A.4.5.2.4.1	Test Purpose and Environment	610

A.4.5.2.4.2	Test Requirements	613

A.4.5.2.5	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	613

A.4.5.2.5.1	Test Purpose and Environment	613

A.4.5.2.5.2	Test Requirements	617

A.4.5.2.6	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	617

A.4.5.2.6.1	Test Purpose and Environment	617

A.4.5.2.6.2	Test Requirements	621

A.4.5.2.7	Void	621

A.4.5.2.8	E-UTRAN - NR FR1 interruptions at NR SRS carrier based switching in asynchronous EN-DC	621

A.4.5.2.8.1	Test Purpose and Environment	621

A.4.5.2.8.2	Test Requirements	624

A.4.5.2.9	E-UTRAN – NR interruptions at E-UTRA SRS carrier based switching	625

A.4.5.2.9.1	Test Purpose and Environment	625

A.4.5.2.9.2	Test Requirements	628

A.4.5.3	SCell Activation and Deactivation Delay	629

A.4.5.3.1	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	629

A.4.5.3.1.1	Test Purpose and Environment	629

A.4.5.3.1.2	Test Requirements	634

A.4.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 320 ms SCell measurement cycle	635

A.4.5.3.2.1	Test Purpose and Environment	635

A.4.5.3.2.2	Test Requirements	635

A.4.5.3.3	SCell Activation and deactivation of unknown SCell in FR1	635

A.4.5.3.3.1	Test Purpose and Environment	635

A.4.5.3.3.2	Test Requirements	636

A.4.5.3.4	SCell Activation and deactivation of multiple unknown SCells in FR1 with single activation/deactivation command	636

A.4.5.3.4.1	Test Purpose and Environment	636

A.4.5.3.3.2	Test Requirements	639

A.4.5.3.5	Direct SCell activation at SCell addition of known SCell in FR1	639

A.4.5.3.5.1	Test Purpose and Environment	639

A.4.5.3.5.2	Test Requirements	644

A.4.5.4	UE UL carrier RRC reconfiguration Delay	644

A.4.5.4.1	UE UL carrier RRC reconfiguration Delay	644

A.4.5.4.1.1 Test Purpose and Environment	644

A.4.5.4.1.2	Test Requirements	654

A.4.5.5	Beam Failure Detection and Link recovery procedures	654

A.4.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	654

A.4.5.5.1.1	Test Purpose and Environment	654

A.4.5.5.1.2	Test Requirements	659

A.4.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	659

A.4.5.5.2.1	Test Purpose and Environment	659

A.4.5.5.2.2	Test Requirements	665

A.4.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	665

A.4.5.5.3.1	Test Purpose and Environment	665

A.4.5.5.3.2	Test Requirements	671

A.4.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in DRX mode	671

A.4.5.5.4.1	Test Purpose and Environment	671

A.4.5.5.4.2	Test Requirements	677

A.4.5.6	Active BWP switch	691

A.4.5.6.1	DCI-based and Timer-based Active BWP Switch	691

A.4.5.6.1.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	691

A.4.5.6.1.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	696

A.4.5.6.2	RRC-based Active BWP Switch	700

A.4.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	705

A.4.5.6.3.1	Simultaneous E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in EN-DC on multiple CCs	705

A.4.5.6.3.1.1	Test Purpose and Environment	705

A.4.5.6.3.1.2	Test Requirements	710

A.4.5.6.4	Simultaneous RRC-based Active BWP Switch on multiple CCs	711

A.4.5.6.4.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC on multiple CCs	711

A.4.5.6.4.1.1	Test Purpose and Environment	711

A.4.5.6.4.1.2	Test Requirements	715

A.4.5.6.5	SCell dormancy switch	715

A.4.5.6.5.1	E-UTRAN – NR FR1 PSCell SCell dormancy switch of single FR1 SCell outside active time	715

A.4.5.6.5.1.1	Test Purpose and Environment	715

A.4.5.6.5.1.2	Test Requirements	721

A.4.5.6.5.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR1 SCells inside active time	721

A.4.5.6.5.2.1	Test Purpose and Environment	721

A.4.5.6.5.2.2	Test Requirements	726

A.4.5.7	PSCell addition and release delay	726

A.4.5.7.1	Addition and Release Delay of known NR PSCell	726

A.4.5.7.1.1	Test purpose and environment	726

A.4.5.7.1.2	Test Requirements	730

A.4.5.8	DL Interruptions at switching between two uplink carriers	730

A.4.5.8.1	Test Purpose and Environment	730

A.4.5.8.2	Test Requirements	734

A.4.6	Measurement procedure	734

A.4.6.1	Intra-frequency Measurements	734

A.4.6.1.1	EN-DC event triggered reporting tests without gap under non-DRX	734

A.4.6.1.1.1	Test purpose and Environment	734

A.4.6.1.1.2	Test parameters	734

A.4.6.1.1.3	Test Requirements	738

A.4.6.1.2	EN-DC event triggered reporting tests without gap under DRX	738

A.4.6.1.2.1	Test purpose and Environment	738

A.4.6.1.2.2	Test parameters	738

A.4.6.1.2.2	Test Requirements	742

A.4.6.1.3	EN-DC event triggered reporting tests with per-UE gaps under non-DRX	742

A.4.6.1.3.1	Test purpose and Environment	742

A.4.6.1.3.2	Test parameters	742

A.4.6.1.3.3	Test Requirements	746

A.4.6.1.4	EN-DC event triggered reporting tests with per-UE gaps under DRX	746

A.4.6.1.4.1	Test purpose and Environment	746

A.4.6.1.4.2	Test parameters	746

A.4.6.1.4.3	Test Requirements	750

A.4.6.1.5	EN-DC event triggered reporting tests without gap under non-DRX with SSB index reading	750

A.4.6.1.5.1	Test purpose and Environment	750

A.4.6.1.5.2	Test parameters	750

A.4.6.1.5.3	Test Requirements	752

A.4.6.1.6	EN-DC event triggered reporting tests with SSB index reading with per-UE gaps	753

A.4.6.1.6.1	Test purpose and Environment	753

A.4.6.1.6.2	Test parameters	753

A.4.6.1.6.3	Test Requirements	755

A.4.6.1.7	EN-DC event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	756

A.4.6.1.7.1	Test purpose and Environment	756

A.4.6.1.7.2	Test parameters	756

A.4.6.1.7.3	Test Requirements	760

A.4.6.2	Inter-frequency Measurements	760

A.4.6.2.1	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	760

A.4.6.2.1.1	Test Purpose and Environment	760

A.4.6.2.1.2	Test Requirements	764

A.4.6.2.2	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	764

A.4.6.2.2.1	Test Purpose and Environment	764

A.4.6.2.2.2	Test Requirements	768

A.4.6.2.3	Void	769

A.4.6.2.4	Void	769

A.4.6.2.5	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	769

A.4.6.2.5.1	Test Purpose and Environment	769

A.4.6.2.5.2	Test Requirements	773

A.4.6.2.6	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	773

A.4.6.2.6.1	Test Purpose and Environment	773

A.4.6.2.6.2	Test Requirements	778

A.4.6.2.7	Void	779

A.4.6.2.8	Void	779

A.4.6.3	Void	779

A.4.6.4	L1-RSRP measurement for beam reporting	779

A.4.6.4.1	SSB based L1-RSRP measurement when DRX is not used	779

A.4.6.4.1.1	Test Purpose and Environment	779

A.4.6.4.1.2	Test parameters	779

A.4.6.4.1.3	Test Requirements	782

A.4.6.4.2	SSB based L1-RSRP measurement when DRX is used	782

A.4.6.4.2.1	Test Purpose and Environment	782

A.4.6.4.2.2	Test parameters	783

A.4.6.4.2.3	Test Requirements	786

A.4.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	786

A.4.6.4.3.1	Test Purpose and Environment	786

A.4.6.4.3.2	Test parameters	787

A.4.6.4.3.3	Test Requirements	790

A.4.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	790

A.4.6.4.4.1	Test Purpose and Environment	790

A.4.6.4.4.2	Test parameters	791

A.4.6.4.4.3	Test Requirements	793

A.4.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	793

A.4.6.4.5.1	Test Purpose and Environment	793

A.4.6.4.5.2	Test parameters	794

A.4.6.4.2.3	Test Requirements	797

A.4.6.5	CLI measurements	797

A.4.6.5.1	SRS-RSRP measurement with non-DRX	797

A.4.6.5.1.1	Test Purpose and Environment	797

A.4.6.5.1.2	Test Parameters	798

A.4.6.5.1.3	Test Requirements	801

A.4.6.5.2	CLI-RSSI measurement with non-DRX	801

A.4.6.5.2.1	Test Purpose and Environment	801

A.4.6.5.2.2	Test Parameters	802

A.4.6.5.2.3	Test Requirements	803

A.4.6.6.1.2	Test Requirements	808

A.4.6.7	L1-SINR measurement for beam reporting	808

A.4.6.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	811

A.4.6.7.2.1	Test Purpose and Environment	811

A.4.6.7.2.2	Test parameters	811

A.4.6.7.2.3	Test Requirements	814

A.4.6.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	814

A.4.6.7.3.1	Test Purpose and Environment	814

A.4.6.7.3.2	Test parameters	814

A.4.6.7.3.3	Test Requirements	816

A.4.6.8	CSI-RS based intra-frequency Measurement	817

A.4.6.8.1	EN-DC event triggered reporting tests without gap under DRX	817

A.4.6.8.1.1	Test purpose and Environment	817

A.4.6.8.1.2	Test Requirements	821

A.4.6.9	CSI-RS based inter-frequency Measurement	821

A.4.6.9.1	EN-DC event triggered reporting tests for FR1 cell when non-DRX is used	821

A.4.6.9.1.1	Test Purpose and Environment	821

A.4.6.9.1.2	Test Requirements	825

A.4.7	Measurement Performance requirements	826

A.4.7.1	SS-RSRP	826

A.4.7.1.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	826

A.4.7.1.1.1	Test Purpose and Environment	826

A.4.7.1.1.2	Test parameters	826

A.4.7.1.1.3	Test Requirements	830

A.4.7.1.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	830

A.4.7.1.2.1	Test Purpose and Environment	830

A.4.7.1.2.2	Test parameters	831

A.4.7.1.2.3	Test Requirements	835

A.4.7.1.3	Void	835

A.4.7.2	SS-RSRQ	835

A.4.7.2.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	835

A.4.7.2.1.1	Test Purpose and Environment	835

A.4.7.2.1.2	Test Parameters	836

A.4.7.2.1.3	Test Requirements	841

A.4.7.2.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	841

A.4.7.2.2.1	Test Purpose and Environment	841

A.4.7.2.2.2	Test Parameters	841

A.4.7.2.2.3	Test Requirements	846

A.4.7.3	SS-SINR	846

A.4.7.3.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	846

A.4.7.3.1.1	Test Purpose and Environment	846

A.4.7.3.1.2	Test Parameters	846

A.4.7.3.1.3	Test Requirements	850

A.4.7.3.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	850

A.4.7.3.2.1	Test Purpose and Environment	850

A.4.7.3.2.2	Test Parameters	850

A.4.7.3.2.3	Test Requirements	855

A.4.7.4	L1-RSRP measurement for beam reporting	855

A.4.7.4.1	SSB based L1-RSRP measurement	855

A.4.7.4.1.1	Test Purpose and Environment	855

A.4.7.4.1.2	Test parameters	856

A.4.7.4.1.3	Test Requirements	859

A.4.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	859

A.4.7.4.2.1	Test Purpose and Environment	859

A.4.7.4.2.2	Test parameters	860

A.4.7.4.2.3	Test Requirements	863

A.4.7.5	SFTD accuracy	863

A.4.7.5.1	SFTD accuracy	863

A.4.7.5.1.1	Test Purpose and Environment	863

A.4.7.5.1.2	Test Parameters	863

A.4.7.5.1.3	Test Requirements	868

A.4.7.5.2	Void	868

A.4.7.5.3	Void	868

A.4.7.6	CLI measurements	868

A.4.7.6.1	EN-DC SRS-RSRP measurement accuracy with FR1 serving cell	868

A.4.7.6.1.1	Test Purpose and Environment	868

A.4.7.6.1.2	Test parameters	869

A.4.7.6.1.3	Test Requirements	874

A.4.7.6.2	EN-DC CLI-RSSI measurement accuracy with FR1 serving cell	874

A.4.7.6.2.1	Test Purpose and Environment	874

A.4.7.6.2.2	Test parameters	875

A.4.7.6.2.3	Test Requirements	878

A.4.7.7	L1-SINR measurement for beam reporting	878

A.4.7.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR	882

A.4.7.7.2.1	Test Purpose and Environment	882

A.4.7.7.2.2	Test parameters	882

A.4.7.7.2.3	Test Requirements	886

A.4.7.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	886

A.4.7.7.3.1	Test Purpose and Environment	886

A.4.7.7.3.2	Test parameters	886

A.4.7.7.3.3	Test Requirements	889

A.4.7.8	CSI-RSRP	889

A.4.7.8.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	889

A.4.7.8.1.1	Test Purpose and Environment	889

A.4.7.8.1.2	Test parameters	890

A.4.7.8.1.3	Test Requirements	895

A.4.7.8.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	895

A.4.7.8.2.1	Test Purpose and Environment	895

A.4.7.8.2.2	Test parameters	895

A.4.7.8.2.3	Test Requirements	900

A.4.7.9	CSI-RSRQ	900

A.4.7.9.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	900

A.4.7.9.1.1	Test Purpose and Environment	900

A.4.7.9.1.2	Test Parameters	900

A.4.7.9.1.3	Test Requirements	905

A.4.7.9.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	905

A.4.7.9.2.1	Test Purpose and Environment	905

A.4.7.9.2.2	Test Parameters	905

A.4.7.9.2.3	Test Requirements	910

A.4.7.10	CSI-SINR	910

A.4.7.10.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	910

A.4.7.10.1.1	Test Purpose and Environment	910

A.4.7.10.1.2	Test Parameters	911

A.4.7.10.1.3	Test Requirements	915

A.4.7.10.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	915

A.4.7.10.2.1	Test Purpose and Environment	915

A.4.7.10.2.2	Test Parameters	916

A.4.7.10.2.3	Test Requirements	920

A.4.8	Void	920

A.5	EN-DC tests with one or more NR cells in FR2	921

A.5.1	Void	921

A.5.2	Void	921

A.5.3	RRC_CONNECTED state mobility	921

A.5.3.1	Void	921

A.5.3.2	RRC Connection Mobility Control	921

A.5.3.2.1	Void	921

A.5.3.2.2	Random Access	921

A.5.3.2.2.1	4-step RA type c ontention based random access test in FR2 for PSCell/SCell in EN-DC	921

A.5.3.2.2.2	4-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	925

A.5.3.2.2.3	2-step RA type contention based random access test in FR2 for PSCell/SCell in EN-DC	929

A.5.3.2.2.4	2-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	933

A.5.3.2.3	Void	937

A.5.4	Timing	937

A.5.4.1	UE transmit timing	937

A.5.4.1.1	NR UE Transmit Timing Test for FR2	937

A.5.4.1.1.1	Test Purpose and environment	937

A.5.4.1.1.2	Test requirements	940

A.5.4.2	UE timer accuracy	941

A.5.4.3	Timing advance	941

A.5.4.3.1 EN-DC FR2 timing advance adjustment accuracy	941

A.5.4.3.1.1 Test Purpose and Environment	941

A.5.4.3.1.2 Test Parameters	941

A.5.4.3.1.3	Test Requirements	944

A.5.5	Signaling characteristics	944

A.5.5.1	Radio link Monitoring	944

A.5.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	944

A.5.5.1.1.1	Test Purpose and Environment	944

A.5.5.1.1.2	Test Requirements	947

A.5.5.1.2	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	948

A.5.5.1.2.1	Test Purpose and Environment	948

A.5.5.1.2.2	Test Requirements	951

A.5.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	952

A.5.5.1.3.1	Test Purpose and Environment	952

A.5.5.1.3.2	Test Requirements	955

A.5.5.1.4	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	955

A.5.5.1.4.1	Test Purpose and Environment	955

A.5.5.1.4.2	Test Requirements	960

A.5.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	960

A.5.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	966

A.5.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	971

A.5.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	977

A.5.5.1.8.2	Test Requirements	983

A.5.5.1.9	EN-DC Radio Link Monitoring UE Scheduling Restrictions on FR2	984

A.5.5.1.9.1	Test Purpose and Environment	984

A.5.5.1.9.2	Test Requirements	986

A.5.5.2	Interruption	986

A.5.5.2.1	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	986

A.5.5.2.1.1	Test Purpose and Environment	986

A.5.5.2.1.2	Test Requirements	989

A.5.5.2.2	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	989

A.5.5.2.2.1	Test Purpose and Environment	989

A.5.5.2.2.2	Test Requirements	992

A.5.5.2.3	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	992

A.5.5.2.3.1	Test Purpose and Environment	992

A.5.5.2.3.2	Test Requirements	996

A.5.5.2.4	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	996

A.5.5.2.4.1	Test Purpose and Environment	996

A.5.5.2.4.2	Test Requirements	999

A.5.5.2.5	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	1000

A.5.5.2.5.1	Test Purpose and Environment	1000

A.5.5.2.5.2	Test Requirements	1003

A.5.5.2.6	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	1004

A.5.5.2.6.1	Test Purpose and Environment	1004

A.5.5.2.6.2	Test Requirements	1006

A.5.5.2.7	E-UTRAN – NR FR2 interruptions at E-UTRA SRS carrier based switching	1007

A.5.5.2.7.1	Test Purpose and Environment	1007

A.5.5.2.7.2	Test Requirements	1009

A.5.5.2.8 E-UTRAN – NR FR2 interruptions at NR SRS carrier based switching	1010

A.5.5.2.8.1 Test Purpose and Environment	1010

A.5.5.2.8.3	Test Requirements	1012

A.5.5.3	SCell Activation and Deactivation Delay	1012

A.5.5.3.1	SCell Activation and deactivation of SCell in FR2 intra-band	1012

A.5.5.3.1.1	Test Purpose and Environment	1012

A.5.5.3.1.2	Test Requirements	1014

A.5.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	1014

A.5.5.3.2.1	Test Purpose and Environment	1014

A.5.5.3.2.2	Test Requirements	1017

A.5.5.3.3	Void	1017

A.5.5.3.4	Void	1017

A.5.5.3.5	SCell Activation and deactivation of SCell in FR2	1017

A.5.5.3.5.1	Test Purpose and Environment	1017

A.5.5.3.5.2	Test Requirements	1021

A.5.5.3.6	Multiple SCell Activation and deactivation of one unknown SCell and one known SCell in FR2	1022

A.5.5.3.6.1	Test Purpose and Environment	1022

A.5.5.3.6.2	Test Requirements	1025

A.5.5.3.7	Direct SCell activation at SCell addition of known SCell in FR2	1026

A.5.5.3.7.1	Test Purpose and Environment	1026

A.5.5.3.7.2	Test Requirements	1029

A.5.5.4	Void	1029

A.5.5.5	Beam Failure Detection and Link recovery procedures	1029

A.5.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1029

A.5.5.5.1.1	Test Purpose and Environment	1029

A.5.5.5.1.2	Test Requirements	1035

A.5.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode	1035

A.5.5.5.2.1	Test Purpose and Environment	1035

A.5.5.5.2.2	Test Requirements	1041

A.5.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	1041

A.5.5.5.3.1	Test Purpose and Environment	1041

A.5.5.5.3.2	Test Requirements	1047

A.5.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1047

A.5.5.5.4.1	Test Purpose and Environment	1047

A.5.5.5.4.2	Test Requirements	1053

A.5.5.5.5	EN-DC scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1053

A.5.5.5.5.1	Test Purpose and Environment	1053

A.5.5.5.5.2	Test Requirements	1059

A.5.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	1059

A.5.5.5.6.1	Test Purpose and Environment	1059

A.5.5.5.6.2	Test Requirements	1064

A.5.5.5.7	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	1064

A.5.5.5.7.1	Test Purpose and Environment	1064

A.5.5.5.7.2	Test Requirements	1070

A.5.5.6	Active BWP switch	1070

A.5.5.6.1	DCI-based and Timer-based Active BWP Switch	1070

A.5.5.6.1.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	1070

A.5.5.6.1.1.1	Test Purpose and Environment	1070

A.5.5.6.1.1.2	Test Requirements	1074

A.5.5.6.1.2	E-UTRAN – NR PSCell FR2 DL active BWP switch with FR2 SCell in non-DRX in synchronous EN-DC	1075

A.5.5.6.2	RRC-based Active BWP Switch	1079

A.5.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1083

A.5.5.6.3.1	E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs in synchronous EN-DC	1083

A.5.5.6.3.1.1	Test Purpose and Environment	1083

A.5.5.6.3.1.2	Test Requirements	1087

A.5.5.6.4	SCell dormancy switch	1088

A.5.5.6.4.1	E-UTRAN – NR FR2 PSCell SCell dormancy switch of single FR2 SCell inside active time	1088

A.5.5.6.4.1.1	Test Purpose and Environment	1088

A.5.5.6.4.1.2	Test Requirements	1092

A.5.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR2 SCells outside active time	1092

A.5.5.6.4.2.1	Test Purpose and Environment	1092

A.5.5.6.4.2.2	Test Requirements	1099

A.5.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1099

A.5.5.6.5.1	E-UTRAN – NR PSCell FR2  and NR SCell FR2 DL active BWP switch on multiple CCs with non-DRX in synchronous EN-DC	1099

A.5.5.7	PSCell addition and release delay	1103

A.5.5.7.1	Addition and Release Delay of NR PSCell	1103

A.5.5.7.1.1	Test purpose and environment	1103

A.5.5.7.1.2	Test Requirements	1106

A.5.5.8	Active TCI state switch delay	1107

A.5.5.8.1	MAC-CE based active TCI state switch	1107

A.5.5.8.1.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1107

A.5.5.8.1.1.1	Test Purpose and Environment	1107

A.5.5.8.1.1.2	Test Requirements	1109

A.5.5.8.2	RRC based active TCI state switch	1109

A.5.5.8.2.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1109

A.5.5.8.2.1.1	Test Purpose and Environment	1109

A.5.5.8.2.1.2	Test Requirements	1112

A.5.5.9	Uplink spatial relation switch delay	1112

A.5.5.9.1	MAC-CE based uplink spatial relation switch	1112

A.5.5.9.1.1	E-UTRAN – NR PSCell FR2 uplink spatial relation switch for a known spatial relation	1112

A.5.5.9.1.1.1	Test Purpose and Environment	1112

A.5.5.9.1.1.2	Test Requirements	1115

A.5.5.9.2	RRC based spatial relation switch	1115

A.5.5.9.2.1	E-UTRAN – NR PSCell FR2 spatial relation switch associated with a known DL-RS	1115

A.5.5.9.2.1.1	Test Purpose and Environment	1115

A.5.5.9.2.1.2	Test Requirements	1118

A.5.5.10	UE specific CBW change	1118

A.5.5.10.1	UE specific CBW change on FR2 NR PSCell	1118

A.5.5.10.1.1	Test Purpose and Environment	1118

A.5.5.10.1.2	Test Requirements	1121

A.5.6	Measurement procedure	1122

A.5.6.1	Intra-frequency Measurements	1122

A.5.6.1.1	EN-DC event triggered reporting test without gap under non-DRX	1122

A.5.6.1.1.1	Test purpose and Environment	1122

A.5.6.1.1.2	Test Requirements	1126

A.5.6.1.2	EN-DC event triggered reporting test without gap under DRX	1126

A.5.6.1.2.1	Test purpose and Environment	1126

A.5.6.1.2.2	Test Requirements	1128

A.5.6.1.3	EN-DC event triggered reporting test with per-UE gaps under non-DRX	1129

A.5.6.1.3.1	Test purpose and Environment	1129

A.5.6.1.3.2	Test Requirements	1133

A.5.6.1.4	EN-DC event triggered reporting test with per-UE gaps under DRX	1133

A.5.6.1.4.1	Test purpose and Environment	1133

A.5.6.1.4.2	Test Requirements	1136

A.5.6.2	Inter-frequency Measurements	1137

A.5.6.2.1 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	1137

A.5.6.2.1.1	Test Purpose and Environment	1137

A.5.6.2.1.2	Test Requirements	1140

A.5.6.2.2 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	1140

A.5.6.2.2.1	Test Purpose and Environment	1140

A.5.6.2.2.2	Test Requirements	1144

A.5.6.2.3 	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	1144

A.5.6.2.3.1	Test Purpose and Environment	1144

A.5.6.2.3.2	Test Requirements	1148

A.5.6.2.4	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	1148

A.5.6.2.4.1	Test Purpose and Environment	1148

A.5.6.2.4.2	Test Requirements	1151

A.5.6.2.5	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	1151

A.5.6.2.5.1	Test Purpose and Environment	1151

A.5.6.2.5.2	Test Requirements	1156

A.5.6.2.6	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	1156

A.5.6.2.6.1	Test Purpose and Environment	1156

A.5.6.2.6.2	Test Requirements	1160

A.5.6.2.7	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	1160

A.5.6.2.7.1	Test Purpose and Environment	1160

A.5.6.2.7.2	Test Requirements	1165

A.5.6.2.8	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	1165

A.5.6.2.8.1	Test Purpose and Environment	1165

A.5.6.2.8.2	Test Requirements	1170

A.5.6.3	L1-RSRP measurement for beam reporting	1171

A.5.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1171

A.5.6.3.1.1	Test Purpose and Environment	1171

A.5.6.3.1.2	Test parameters	1171

A.5.6.3.1.3	Test Requirements	1173

A.5.6.3.2	SSB based L1-RSRP measurement when DRX is used	1173

A.5.6.3.2.1	Test Purpose and Environment	1173

A.5.6.3.2.2	Test parameters	1174

A.5.6.3.2.3	Test Requirements	1176

A.5.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	1177

A.5.6.3.3.1	Test Purpose and Environment	1177

A.5.6.3.3.2	Test parameters	1177

A.5.6.3.3.3	Test Requirements	1179

A.5.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	1180

A.5.6.3.4.1	Test Purpose and Environment	1180

A.5.6.3.4.2	Test parameters	1180

A.5.6.3.4.3	Test Requirements	1182

A.5.6.4	CLI measurements	1183

A.5.6.4.1	SRS-RSRP measurement with DRX	1183

A.5.6.4.1.1	Test Purpose and Environment	1183

A.5.6.4.1.2	Test Parameters	1183

A.5.6.4.1.3	Test Requirements	1185

A.5.6.4.2	CLI-RSSI measurement with DRX	1185

A.5.6.4.2.1	Test Purpose and Environment	1185

A.5.6.4.2.2	Test Parameters	1186

A.5.6.4.2.3	Test Requirements	1188

A.5.6.5	Measurements with autonomous gaps	1188

A.5.6.5.1 	EN-DC inter-frequency CGI identification of NR neighbor cell in FR2	1188

A.5.6.5.1.1	Test Purpose and Environment	1188

A.5.6.5.1.2	Test Requirements	1191

A.5.6.6	L1-SINR measurement for beam reporting	1192

A.5.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	1194

A.5.6.6.2.1	Test Purpose and Environment	1194

A.5.6.6.2.2	Test parameters	1195

A.5.6.6.2.3	Test Requirements	1198

A.5.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	1198

A.5.6.6.3.1	Test Purpose and Environment	1198

A.5.6.6.3.2	Test parameters	1198

A.5.6.6.3.3	Test Requirements	1200

A.5.6.7	CSI-RS based Intra-frequency Measurements	1200

A.5.6.7.1	EN-DC event triggered reporting test without gap under non-DRX	1200

A.5.6.7.1.1	Test purpose and Environment	1200

A.5.6.7.1.2	Test Requirements	1203

A.5.6.8	CSI-RS based Inter-frequency Measurements	1203

A.5.6.8.1 	EN-DC event triggered reporting tests for NR FR2 cell when DRX is used	1203

A.5.6.8.1.1	Test Purpose and Environment	1203

A.5.6.8.1.2	Test Requirements	1208

A.5.7	Measurement Performance requirements	1208

A.5.7.1	SS-RSRP	1208

A.5.7.1.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1208

A.5.7.1.1.1	Test Purpose and Environment	1208

A.5.7.1.1.2	Test parameters	1209

A.5.7.1.1.3	Test Requirements	1211

A.5.7.1.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1212

A.5.7.1.2.1	Test Purpose and Environment	1212

A.5.7.1.2.2	Test parameters	1212

A.5.7.1.2.3	Test Requirements	1216

A.5.7.1.3	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	1217

A.5.7.1.3.1	Test Purpose and Environment	1217

A.5.7.1.3.2	Test parameters	1217

A.5.7.1.3.3	Test Requirements	1220

A.5.7.2	SS-RSRQ	1221

A.5.7.2.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1221

A.5.7.2.1.1	Test Purpose and Environment	1221

A.5.7.2.1.2	Test Parameters	1221

A.5.7.2.1.3	Test Requirements	1225

A.5.7.2.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1225

A.5.7.2.2.1	Test Purpose and Environment	1225

A.5.7.2.2.2	Test Parameters	1225

A.5.7.2.2.3	Test Requirements	1228

A.5.7.3	SS-SINR	1228

A.5.7.3.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1228

A.5.7.3.1.1	Test Purpose and Environment	1228

A.5.7.3.1.2	Test Parameters	1228

A.5.7.3.1.3	Test Requirements	1231

A.5.7.3.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1231

A.5.7.3.2.1	Test Purpose and Environment	1231

A.5.7.3.2.2	Test Parameters	1231

A.5.7.3.2.3	Test Requirements	1233

A.5.7.4	L1-RSRP measurement for beam reporting	1234

A.5.7.4.1	SSB based L1-RSRP measurement	1234

A.5.7.4.1.1	Test Purpose and Environment	1234

A.5.7.4.1.2	Test parameters	1234

A.5.7.4.1.3	Test Requirements	1236

A.5.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1237

A.5.7.4.2.1	Test Purpose and Environment	1237

A.5.7.4.2.2	Test parameters	1237

A.5.7.4.2.3	Test Requirements	1239

A.5.7.5	CLI measurements	1240

A.5.7.5.1	EN-DC SRS-RSRP measurement accuracy with FR2 serving cell	1240

A.5.7.5.1.1	Test Purpose and Environment	1240

A.5.7.5.1.2	Test parameters	1240

A.5.7.5.1.3	Test Requirements	1243

A.5.7.5.2	EN-DC CLI-RSSI measurement accuracy with FR2 serving cell	1244

A.5.7.5.2.1	Test Purpose and Environment	1244

A.5.7.5.2.2	Test parameters	1244

A.5.7.5.2.3	Test Requirements	1246

A.5.7.6	L1-SINR measurement for beam reporting	1247

A.5.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	1249

A.5.7.6.2.1	Test Purpose and Environment	1249

A.5.7.6.2.2	Test parameters	1250

A.5.7.6.2.3	Test Requirements	1252

A.5.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	1253

A.5.7.6.3.1	Test Purpose and Environment	1253

A.5.7.6.3.2	Test parameters	1253

A.5.7.6.3.3	Test Requirements	1255

A.5.7.7	CSI-RSRP	1255

A.5.7.7.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1255

A.5.7.7.1.1	Test Purpose and Environment	1255

A.5.7.7.1.2	Test parameters	1256

A.5.7.7.1.3	Test Requirements	1258

A.5.7.7.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1259

A.5.7.7.2.1	Test Purpose and Environment	1259

A.5.7.7.2.2	Test parameters	1259

A.5.7.7.2.3	Test Requirements	1263

A.5.7.8	CSI-RSRQ	1264

A.5.7.8.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	1264

A.5.7.8.1.1	Test Purpose and Environment	1264

A.5.7.8.1.2	Test Parameters	1264

A.5.7.8.1.3	Test Requirements	1266

A.5.7.8.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1266

A.5.7.8.2.1	Test Purpose and Environment	1266

A.5.7.8.2.2	Test Parameters	1266

A.5.7.8.2.3	Test Requirements	1268

A.5.7.9	CSI-SINR	1268

A.5.7.9.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1268

A.5.7.9.1.1	Test Purpose and Environment	1268

A.5.7.9.1.2	Test Parameters	1268

A.5.7.9.1.3	Test Requirements	1271

A.5.7.9.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1271

A.5.7.9.2.1	Test Purpose and Environment	1271

A.5.7.9.2.2	Test Parameters	1271

A.5.7.9.2.3	Test Requirements	1274

A.5.8	Void	1274

A.6	NR standalone tests with all NR cells in FR1	1011

A.6.1	SA: RRC_IDLE state mobility	1011

A.6.1.1	Cell re-selection to NR	1011

A.6.1.1.1	Cell reselection to FR1 intra-frequency NR case	1011

A.6.1.1.1.1	Test Purpose and Environment	1011

A.6.1.1.1.2	Test Parameters	1011

A.6.1.1.1.3	Test Requirements	1015

A.6.1.1.2	Cell reselection to FR1 inter-frequency NR case	1015

A.6.1.1.2.1	Test Purpose and Environment	1015

A.6.1.1.2.2	Test Parameters	1015

A.6.1.1.2.3	Test Requirements	1019

A.6.1.1.3	Cell reselection to FR1 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1019

A.6.1.1.3.1	Test Purpose and Environment	1019

A.6.1.1.3.2	Test Parameters	1019

A.6.1.1.3.3	Test Requirements	1024

A.6.1.1.4	Cell reselection to FR1 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1024

A.6.1.1.4.1	Test Purpose and Environment	1024

A.6.1.1.4.2	Test Parameters	1024

A.6.1.1.4.3	Test Requirements	1027

A.6.1.1.5	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1027

A.6.1.1.5.1	Test Purpose and Environment	1027

A.6.1.1.5.2	Test Parameters	1027

A.6.1.1.5.3	Test Requirements	1032

A.6.1.1.6	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1032

A.6.1.1.6.1	Test Purpose and Environment	1032

A.6.1.1.6.2	Test Parameters	1033

A.6.1.1.6.3	Test Requirements	1037

A.6.1.1.7	Cell reselection to FR1 intra-frequency NR case for UE configured with highSpeedMeasFlag-r16	1038

A.6.1.1.7.1	Test Purpose and Environment	1038

A.6.1.1.7.2	Test Parameters	1038

A.6.1.1.7.3	Test Requirements	1041

A.6.1.2	Inter-RAT E-UTRAN cell re-selection	1041

A.6.1.2.1	Cell reselection to higher priority E-UTRAN	1041

A.6.1.2.1.1	Test Purpose and Environment	1041

A.6.1.2.1.2	Test Parameters	1041

A.6.1.2.1.3	Test Requirements	1044

A.6.1.2.2	Cell reselection to lower priority E-UTRAN	1045

A.6.1.2.2.1	Test Purpose and Environment	1045

A.6.1.2.2.2	Test Parameters	1045

A.6.1.2.2.3	Test Requirements	1048

A.6.1.2.3	Cell reselection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion	1049

A.6.1.2.3.1	Test Purpose and Environment	1049

A.6.1.2.3.2	Test Parameters	1049

A.6.1.2.3.3	Test Requirements	1052

A.6.1.2.4	Cell reselection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion	1053

A.6.1.2.4.1	Test Purpose and Environment	1053

A.6.1.2.4.2	Test Parameters	1053

A.6.1.2.4.3	Test Requirements	1057

A.6.1.1.7	Void	1061

A.6.2	SA: RRC_INACTIVE state mobility	1061

A.6.3	RRC_CONNECTED state mobility	1061

A.6.3.1	Handover	1061

A.6.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell	1061

A.6.3.1.1.1	Test Purpose and Environment	1061

A.6.3.1.1.2	Test Parameters	1061

A.6.3.1.1.3 Test Requirements	1064

A.6.3.1.2	Intra-frequency handover from FR1 to FR1; unknown target cell	1064

A.6.3.1.2.1	Test Purpose and Environment	1064

A.6.3.1.2.2	Test Parameters	1064

A.6.3.1.2.3	Test Requirements	1068

A.6.3.1.3	Inter-frequency handover from FR1 to FR1; unknown target cell	1068

A.6.3.1.3.1	Test Purpose and Environment	1068

A.6.3.1.3.2	Test Parameters	1068

A.6.3.1.3.3 Test Requirements	1072

A.6.3.1.4	 SA NR - E-UTRAN handover	1072

A.6.3.1.4.1	Test Purpose and Environment	1072

A.6.3.1.4.2	Test Requirements	1078

A.6.3.1.5	SA NR - E-UTRAN handover with unknown target cell	1078

A.6.3.1.5.1	Test Purpose and Environment	1078

A.6.3.1.5.2	Test Requirements	1084

A.6.3.1.6	 SA NR - UTRAN FDD handover	1084

A.6.3.1.6.1	Test Purpose and Environment	1084

A.6.3.1.6.2	Test Requirements	1088

A.6.3.1.7	Intra-frequency handover in FR1; synchronous scenario	1088

A.6.3.1.7.1	Test Purpose and Environment	1088

A.6.3.1.7.2	Test Parameters	1088

A.6.3.1.7.3 Test Requirements	1092

A.6.3.1.8	Intra-frequency handover in FR1; asynchronous scenario	1093

A.6.3.1.8.1	Test Purpose and Environment	1093

A.6.3.1.8.2	Test Parameters	1093

A.6.3.1.8.3 Test Requirements	1097

A.6.3.1.9	Intra-band inter-frequency synchronous DAPS handover test in SA for FR1	1098

A.6.3.1.9.1	Test Purpose and Environment	1098

A.6.3.1.9.2	Test Parameters	1098

A.6.3.1.9.3 Test Requirements	1102

A.6.3.1.10	Intra-band inter-frequency asynchronous DAPS handover test in SA for FR1	1102

A.6.3.1.10.1	Test Purpose and Environment	1102

A.6.3.1.10.2	Test Parameters	1102

A.6.3.1.10.3 Test Requirements	1106

A.6.3.1.11	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR1	1106

A.6.3.1.11.1	Test Purpose and Environment	1106

A.6.3.1.11.2	Test Parameters	1106

A.6.3.1.11.3 Test Requirements	1113

A.6.3.1.12	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR1	1113

A.6.3.1.12.1	Test Purpose and Environment	1113

A.6.3.1.12.2	Test Parameters	1113

A.6.3.1.12.3 Test Requirements	1120

A.6.3.2	RRC Connection Mobility Control	1120

A.6.3.2.1	SA: RRC Re-establishment	1120

A.6.3.2.1.1	Intra-frequency RRC Re-establishment in FR1	1120

A.6.3.2.1.2	Inter-frequency RRC Re-establishment in FR1	1124

A.6.3.2.1.3	Intra-frequency RRC Re-establishment in FR1 without serving cell timing	1128

A.6.3.2.2	Random Access	1131

A.6.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	1131

A.6.3.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone	1135

A.6.3.2.2.3	2-step RA type contention based random access test in FR1 for NR standalone	1140

A.6.3.2.2.4	2-step RA type non-contention based test in FR1 for NR standalone	1145

A.6.3.2.3	SA: RRC Connection Release with Redirection	1149

A.6.3.2.3.1	Redirection from NR in FR1 to NR in FR1	1149

A.6.3.2.3.2	Redirection from NR in FR1 to E-UTRAN	1153

A.6.3.3 Conditional handover	1160

A.6.3.3.1	Intra-frequency conditional handover from FR1 to FR1	1160

A.6.3.3.1.1	Test Purpose and Environment	1160

A.6.3.3.1.2	Test Parameters	1160

A.6.3.3.1.3 Test Requirements	1164

A.6.3.3.2	Inter-frequency conditional handover from FR1 to FR1	1164

A.6.3.3.2.1	Test Purpose and Environment	1164

A.6.3.3.2.2	Test Parameters	1164

A.6.3.3.2.3 Test Requirements	1168

A.6.4	Timing	1168

A.6.4.1	UE transmit timing	1168

A.6.4.1.1	NR UE Transmit Timing Test for FR1	1168

A.6.4.1.1.1	Test Purpose and environment	1168

A.6.4.1.1.2	Test requirements	1172

A.6.4.2	UE timer accuracy	1172

A.6.4.3	Timing advance	1172

A.6.4.3.1	SA FR1 timing advance adjustment accuracy	1172

A.6.4.3.1.1	Test Purpose and Environment	1172

A.6.4.3.1.2	Test Parameters	1172

A.6.4.3.1.3	Test Requirements	1176

A.6.5	Signalling characteristics	1176

A.6.5.1	Radio link Monitoring	1176

A.6.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	1177

A.6.5.1.1.1	Test Purpose and Environment	1177

A.6.5.1.1.2	Test Requirements	1182

A.6.5.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	1182

A.6.5.1.2.1	Test Purpose and Environment	1182

A.6.5.1.2.2	Test Requirements	1188

A.6.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	1188

A.6.5.1.3.1	Test Purpose and Environment	1188

A.6.5.1.3.2	Test Requirements	1194

A.6.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	1194

A.6.5.1.4.1	Test Purpose and Environment	1194

A.6.5.1.4.2	Test Requirements	1200

A.6.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	1200

A.6.5.1.5.1	Test Purpose and Environment	1200

A.6.5.1.5.2	Test Requirements	1206

A.6.5.1.6	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	1206

A.6.5.1.6.1	Test Purpose and Environment	1206

A.6.5.1.6.2	Test Requirements	1212

A.6.5.1.7	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	1212

A.6.5.1.7.1	Test Purpose and Environment	1212

A.6.5.1.7.2	Test Requirements	1217

A.6.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	1217

A.6.5.1.8.1	Test Purpose and Environment	1217

A.6.5.1.8.2	Test Requirements	1223

A.6.5.2	Interruption	1223

A.6.5.2.1	Interruptions during measurements on deactivated NR SCC in FR1	1223

A.6.5.2.1.2	Test Requirements	1227

A.6.5.2.2	SA interruptions at NR SRS carrier based switching	1228

A.6.5.2.2.1	Test Purpose and Environment	1228

A.6.5.2.2.2	Test Parameters	1228

A.6.5.2.2.3	Test Requirements	1230

A.6.5.3	SCell Activation and Deactivation Delay	1230

A.6.5.3.1	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle	1230

A.6.5.3.1.1	Test Purpose and Environment	1230

A.6.5.3.1.2	Test Requirements	1234

A.6.5.3.2	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 320ms SCell measurement cycle	1235

A.6.5.3.2.1	Test Purpose and Environment	1235

A.6.5.3.2.2	Test Requirements	1235

A.6.5.3.3	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	1235

A.6.5.3.3.1	Test Purpose and Environment	1235

A.6.5.3.3.2	Test Requirements	1236

A.6.5.3.4	Direct SCell activation at SCell addition of known SCell in FR1	1236

A.6.5.3.4.1	Test Purpose and Environment	1236

A.6.5.3.4.2	Test Requirements	1240

A.6.5.3.5	Direct SCell activation at handover with known SCell in FR1	1240

A.6.5.3.5.1	Test Purpose and Environment	1240

A.6.5.3.5.2	Test Requirements	1244

A.6.5.4	UE UL carrier RRC reconfiguration Delay	1245

A.6.5.4.1	UE UL carrier RRC reconfiguration Delay	1245

A.6.5.4.1.1	Test Purpose and Environment	1245

A.6.5.4.1.2	Test Requirements	1254

A.6.5.4.2	Void	1255

A.6.5.5	Beam Failure Detection and Link recovery procedures	1255

A.6.5.5.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	1255

A.6.5.5.1.1	Test Purpose and Environment	1255

A.6.5.5.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	1260

A.6.5.5.2.1	Test Purpose and Environment	1260

A.6.5.5.2.2	Test Requirements	1267

A.6.5.5.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	1267

A.6.5.5.3.1	Test Purpose and Environment	1267

A.6.5.5.3.2	Test Requirements	1273

A.6.5.5.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	1273

A.6.5.5.4.1	Test Purpose and Environment	1273

A.6.5.5.4.2	Test Requirements	1279

A.6.5.5.5	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	1279

A.6.5.5.5.1	Test Purpose and Environment	1279

A.6.5.5.5.2	Test Requirements	1283

A.6.5.5.6	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	1284

A.6.5.5.6.1	Test Purpose and Environment	1284

A.6.5.5.6.2	Test Requirements	1288

A.6.5.6	Active BWP switch	1288

A.6.5.6.1	DCI-based and Timer-based Active BWP Switch	1288

A.6.5.6.1.1	NR FR1- NR FR1 DL active BWP switch of PCell with non-DRX in SA	1288

A.6.5.6.1.2	NR FR1 DL active BWP switch with non-DRX in SA	1293

A.6.5.6.2	RRC-based Active BWP Switch	1298

A.6.5.6.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	1298

A.6.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1302

A.6.5.6.3.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	1302

A.6.5.6.3.1.1	Test Purpose and Environment	1302

A.6.5.6.3.1.2	Test Requirements	1307

A.6.5.6.4	SCell dormancy switch	1308

A.6.5.6.4.1	NR FR1 PCell SCell dormancy switch of single FR1 SCell outside active time	1308

A.6.5.6.4.1.1	Test Purpose and Environment	1308

A.6.5.6.4.1.2	Test Requirements	1313

A.6.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR1 SCells inside active time	1314

A.6.5.6.4.2.1	 Test Purpose and Environment	1314

A.6.5.6.4.2.2	 Test Requirements	1318

A.6.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1318

A.6.5.6.5.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	1318

A.6.5.7	DL interruptions at switching between two uplink carriers	1322

A.6.5.7.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	1322

A.6.5.7.1.1	Test Purpose and Environment	1322

A.6.5.7.1.2	Test Requirements	1326

A.6.5.7.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	1326

A.6.5.7.2.1	Test Purpose and Environment	1326

A.6.5.7.2.2	Test Requirements	1330

A.6.5.8.1	UE specific CBW change on PCell in FR1 in non-DRX	1330

A.6.5.8.1.1	Test Purpose and Environment	1330

A.6.5.8.1.2	Test Requirements	1334

A.6.6	Measurement procedure	1334

A.6.6.1	Intra-frequency Measurements	1334

A.6.6.1.1	SA event triggered reporting tests without gap under non-DRX	1334

A.6.6.1.1.1	Test purpose and Environment	1334

A.6.6.1.1.2	Test parameters	1334

A.6.6.1.1.3	Test Requirements	1338

A.6.6.1.2	SA event triggered reporting tests without gap under DRX	1338

A.6.6.1.2.1	Test purpose and Environment	1338

A.6.6.1.2.2	Test parameters	1338

A.6.6.1.2.3	Test Requirements	1342

A.6.6.1.3	SA event triggered reporting tests with per-UE gaps under non-DRX	1342

A.6.6.1.3.1	Test purpose and Environment	1342

A.6.6.1.3.2	Test parameters	1342

A.6.6.1.3.3	Test Requirements	1346

A.6.6.1.4	SA event triggered reporting tests with per-UE gaps under DRX	1346

A.6.6.1.4.1	Test purpose and Environment	1346

A.6.6.1.4.2	Test parameters	1346

A.6.6.1.4.3	Test Requirements	1350

A.6.6.1.5	SA event triggered reporting tests without gap under non-DRX with SSB index reading	1350

A.6.6.1.5.1	Test purpose and Environment	1350

A.6.6.1.5.2	Test parameters	1350

A.6.6.1.5.3	Test Requirements	1352

A.6.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading	1353

A.6.6.1.6.1	Test purpose and Environment	1353

A.6.6.1.6.2	Test parameters	1353

A.6.6.1.6.3	Test Requirements	1354

A.6.6.1.7	SA event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	1355

A.6.6.1.7.1	Test purpose and Environment	1355

A.6.6.1.7.2	Test parameters	1355

A.6.6.1.7.3	Test Requirements	1359

A.6.6.2	Inter-frequency Measurements	1359

A.6.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	1359

A.6.6.2.1.1	Test Purpose and Environment	1359

A.6.6.2.1.2	Test Requirements	1363

A.6.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used	1363

A.6.6.2.2.1	Test Purpose and Environment	1363

A.6.6.2.2.2	Test Requirements	1367

A.6.6.2.3	Void	1368

A.6.6.2.4	Void	1368

A.6.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	1368

A.6.6.2.5.1	Test Purpose and Environment	1368

A.6.6.2.5.2	Test Requirements	1372

A.6.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used	1372

A.6.6.2.6.1	Test Purpose and Environment	1372

A.6.6.2.6.2	Test Requirements	1376

A.6.6.2.7	Void	1377

A.6.6.2.8	Void	1377

A.6.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern	1377

A.6.6.2.9.1	Test Purpose and Environment	1377

A.6.6.2.9.2	Test Requirements	1380

A.6.6.2.10	SA event triggered reporting tests for FR1 when DRX is used	1380

A.6.6.2.10.1	Test Purpose and Environment	1380

A.6.6.2.10.2	Test Requirements	1384

A.6.6.3	Inter-RAT Measurements	1388

A.6.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	1388

A.6.6.3.1.1	Test Purpose and Environment	1388

A.6.6.3.1.2	Test Requirements	1394

A.6.6.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	1394

A.6.6.3.2.1	Test Purpose and Environment	1394

A.6.6.3.2.2	Test Requirements	1401

A.6.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for UE configured with highSpeedMeasFlag-r16	1401

A.6.6.3.3.1	Test Purpose and Environment	1401

A.6.6.3.3.2	Test Requirements	1405

A.6.6.4	L1-RSRP measurement for beam reporting	1405

A.6.6.4.1	SSB based L1-RSRP measurement when DRX is not used	1405

A.6.6.4.1.1	Test Purpose and Environment	1405

A.6.6.4.1.2	Test parameters	1406

A.6.6.4.1.3	Test Requirements	1409

A.6.6.4.2	SSB based L1-RSRP measurement when DRX is used	1409

A.6.6.4.2.1	Test Purpose and Environment	1409

A.6.6.4.2.2	Test parameters	1410

A.6.6.4.2.3	Test Requirements	1413

A.6.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	1413

A.6.6.4.3.1	Test Purpose and Environment	1413

A.6.6.4.3.2	Test parameters	1414

A.6.6.4.3.3	Test Requirements	1417

A.6.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	1417

A.6.6.4.4.1	Test Purpose and Environment	1417

A.6.6.4.4.2	Test parameters	1418

A.6.6.4.4.3	Test Requirements	1421

A.6.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	1421

A.6.6.4.5.1	Test Purpose and Environment	1421

A.6.6.4.5.2	Test parameters	1422

A.6.6.4.5.3	Test Requirements	1424

A.6.6.5	1424

A.6.6.5.1	SA NR - UTRAN FDD event-triggered reporting in non-DRX in FR1	1424

A.6.6.5.1.1	Test Purpose and Environment	1424

A.6.6.5.1.2	Test Requirements	1428

A.6.6.6	CLI measurements	1428

A.6.6.6.1	SRS-RSRP measurement with DRX	1428

A.6.6.6.1.1	Test Purpose and Environment	1428

A.6.6.6.1.2	Test Parameters	1429

A.6.6.6.1.3	Test Requirements	1432

A.6.6.6.2	CLI-RSSI measurement with DRX	1432

A.6.6.6.2.1	Test Purpose and Environment	1432

A.6.6.6.2.2	Test Parameters	1433

A.6.6.6.2.3	Test Requirements	1435

A.6.6.7	NR measurements with autonomous gaps	1435

A.6.6.7.1	SA intra-frequency CGI identification of NR neighbor cell in FR1	1435

A.6.6.7.1.1	Test Purpose and Environment	1435

A.6.6.7.1.2	Test Parameters	1435

A.6.6.7.1.3	Test Requirements	1438

A.6.6.7.2	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA	1438

A.6.6.7.2.1	Test Purpose and Environment	1438

A.6.6.7.2.2	Test Requirements	1441

A.6.6.8	L1-SINR measurement for beam reporting	1442

A.6.6.8.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	1444

A.6.6.8.2.1	Test Purpose and Environment	1444

A.6.6.8.2.2	Test parameters	1444

A.6.6.8.2.3	Test Requirements	1449

A.6.6.8.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	1449

A.6.6.8.3.1	Test Purpose and Environment	1449

A.6.6.8.3.2	Test parameters	1450

A.6.6.8.3.3	Test Requirements	1452

A.6.6.9	Idle Mode CA/DC Measurements	1452

A.6.6.9.1	SA Idle mode CA/DC measurement for FR1	1452

A.6.6.9.1.1	Test Purpose and Environment	1452

A.6.6.9.1.2	Test Requirements	1459

A.6.6.10	CSI-RS based intra-frequency Measurements	1459

A.6.6.10.1	SA event triggered reporting tests without gap under non-DRX	1459

A.6.6.10.1.1	Test purpose and Environment	1459

A.6.6.10.1.2	Test Requirements	1462

A.6.6.11	CSI-RS based inter-frequency Measurements	1462

A.6.6.11.1	 SA event triggered reporting tests with gap under DRX	1462

A.6.6.11.1.1	Test Purpose and Environment	1462

A.6.6.11.1.2	Test Requirements	1466

A.6.6.12	RSTD measurements	1466

A.6.6.12.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA	1466

A.6.6.12.1.1	Test Purpose and Environment	1466

A.6.6.12.1.2	Test Requirements	1472

A.6.6.12.2	NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR1 SA	1473

A.6.6.12.2.1	Test Purpose and Environment	1473

A.6.6.12.2.2	Test Requirements	1478

A.6.6.13 PRS-RSRP measurements	1479

A.6.6.13.1	PRS-RSRP reporting delay test case for single positioning frequency layer	1479

A.6.6.13.1.1	Test purpose and Environment	1479

A.6.6.13.1.2	Test Requirements	1482

A.6.6.13.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	1482

A.6.6.13.2.1	Test purpose and Environment	1482

A.6.6.13.2.2	Test Requirements	1485

A.6.6.14	UE Rx-Tx time difference measurements	1485

A.6.6.14.1	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	1485

A.6.6.14.1.1	Test purpose and environment	1485

A.6.6.14.1.2	Test requirements	1488

A.6.6.14.2	UE Rx-Tx time difference measurement for dual positioning frequency layers in FR1 SA	1488

A.6.6.14.2.1	Test purpose and environment	1488

A.6.6.14.2.2	Test requirements	1491

A.6.7	Measurement Performance requirements	1491

A.6.7.1	SS-RSRP	1491

A.6.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1491

A.6.7.1.1.1	Test Purpose and Environment	1491

A.6.7.1.1.2	Test parameters	1492

A.6.7.1.1.3	Test Requirements	1497

A.6.7.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1497

A.6.7.1.2.1	Test Purpose and Environment	1497

A.6.7.1.2.2	Test parameters	1497

A.6.7.1.2.3	Test Requirements	1501

A.6.7.1.3	Void	1501

A.6.7.2	SS-RSRQ	1501

A.6.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1501

A.6.7.2.1.1	Test Purpose and Environment	1501

A.6.7.2.1.2	Test Parameters	1502

A.6.7.2.1.3	Test Requirements	1507

A.6.7.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1507

A.6.7.2.2.1	Test Purpose and Environment	1507

A.6.7.2.2.2	Test Parameters	1507

A.6.7.2.2.3	Test Requirements	1512

A.6.7.3	SS-SINR	1512

A.6.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1512

A.6.7.3.1.1	Test Purpose and Environment	1512

A.6.7.3.1.2	Test Parameters	1513

A.6.7.3.1.3	Test Requirements	1517

A.6.7.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1517

A.6.7.3.2.1	Test Purpose and Environment	1517

A.6.7.3.2.2	Test Parameters	1518

A.6.7.3.2.3	Test Requirements	1522

A.6.7.4	L1-RSRP measurement for beam reporting	1523

A.6.7.4.1	SSB based L1-RSRP measurement	1523

A.6.7.4.1.1	Test Purpose and Environment	1523

A.6.7.4.1.2	Test parameters	1523

A.6.7.4.1.3	Test Requirements	1527

A.6.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1527

A.6.7.4.2.1	Test Purpose and Environment	1527

A.6.7.4.2.2	Test parameters	1528

A.6.7.4.2.3	Test Requirements	1532

A.6.7.5	E-UTRAN RSRP	1532

A.6.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell	1532

A.6.7.5.1.1	Test Purpose and Environment	1532

A.6.7.5.1.2	Test parameters	1533

A.6.7.5.1.3	Test Requirements	1538

A.6.7.6	E-UTRAN RSRQ	1539

A.6.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell	1539

A.6.7.6.1.1	Test Purpose and Environment	1539

A.6.7.6.1.2	Test parameters	1539

A.6.7.6.1.3	Test Requirements	1544

A.6.7.7	E-UTRAN RS-SINR	1545

A.6.7.7.1	SA: inter-RAT measurement accuracy with FR1 serving cell	1545

A.6.7.7.1.1	Test Purpose and Environment	1545

A.6.7.7.1.2	Test parameters	1545

A.6.7.7.1.3	Test Requirements	1550

A.6.7.8	CLI measurements	1551

A.6.7.8.1	SA SRS-RSRP measurement accuracy with FR1 serving cell	1551

A.6.7.8.1.1	Test Purpose and Environment	1551

A.6.7.8.1.2	Test parameters	1551

A.6.7.8.1.3	Test Requirements	1557

A.6.7.8.2	SA CLI-RSSI measurement accuracy with FR1 serving cell	1557

A.6.7.8.2.1	Test Purpose and Environment	1557

A.6.7.8.2.2	Test parameters	1558

A.6.7.8.2.3	Test Requirements	1561

A.6.7.9	L1-SINR measurement for beam reporting	1562

A.6.7.9.2	L1-SINR measurement with SSB based CMR and dedicated IMR	1566

A.6.7.9.2.1	Test Purpose and Environment	1566

A.6.7.9.2.2	Test parameters	1567

A.6.7.9.2.3	Test Requirements	1572

A.6.7.9.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	1572

A.6.7.9.3.1	Test Purpose and Environment	1572

A.6.7.9.3.2	Test parameters	1572

A.6.7.9.3.3	Test Requirements	1577

A.6.7.10	CSI-RSRP	1577

A.6.7.10.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1577

A.6.7.9.10.1	Test Purpose and Environment	1577

A.6.7.9.10.2	Test parameters	1578

A.6.7.10.1.3	Test Requirements	1583

A.6.7.10.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1583

A.6.7.9.10.1	Test Purpose and Environment	1583

A.6.7.10.2.2	Test parameters	1583

A.6.7.10.2.3	Test Requirements	1588

A.6.7.11	CSI-RSRQ	1588

A.6.7.11.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1588

A.6.7.11.1.1	Test Purpose and Environment	1588

A.6.7.11.1.2	Test Parameters	1588

A.6.7.11.1.3	Test Requirements	1593

A.6.7.11.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1593

A.6.7.11.2.1	Test Purpose and Environment	1593

A.6.7.11.2.2	Test Parameters	1593

A.6.7.11.2.3	Test Requirements	1599

A.6.7.12	CSI-SINR	1600

A.6.7.12.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1600

A.6.7.12.1.1	Test Purpose and Environment	1600

A.6.7.12.1.2	Test Parameters	1600

A.6.7.12.1.3	Test Requirements	1604

A.6.7.12.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1605

A.6.7.12.2.1	Test Purpose and Environment	1605

A.6.7.12.2.2	Test Parameters	1605

A.6.7.12.2.3	Test Requirements	1611

A.6.7.13	RSTD measurements	1611

A.6.7.13.1	RSTD measurement accuracy test case for single positioning frequency layer	1611

A.6.7.13.1.1	Test purpose and Environment	1611

A.6.7.13.1.2	Test Requirements	1614

A.6.7.13.2	RSTD measurement accuracy test case for dual positioning frequency layer	1614

A.6.7.13.2.1	Test purpose and Environment	1614

A.6.7.13.2.2	Test Requirements	1617

A.6.7.14	PRS-RSRP measurements	1617

A.6.7.14.1	SA: measurement accuracy with PRS in FR1	1617

A.6.7.14.1.1	Test Purpose and Environment	1617

A.6.7.14.1.2	Test parameters	1617

A.6.7.14.1.3	Test Requirements	1622

A.6.7.15	UE Rx-Tx time difference measurements	1622

A.6.7.15.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA	1622

A.6.7.15.1.1	Test purpose and environment	1622

A.6.7.15.1.2	Test parameters	1623

A.6.7.15.1.3	Test requirements	1626

A.7	NR standalone tests with one or more NR cells in FR2	1626

A.7.1	SA: RRC_IDLE state mobility	1626

A.7.1.1	Cell re-selection to NR	1626

A.7.1.1.1	Cell reselection to FR2 intra-frequency NR case	1626

A.7.1.1.1.1	Test Purpose and Environment	1626

A.7.1.1.1.2	Test Parameters	1626

A.7.1.1.1.3	Test Requirements	1630

A.7.1.1.2	Cell reselection to FR2 inter-frequency NR case	1630

A.7.1.1.2.1	Test Purpose and Environment	1630

A.7.1.1.2.2	Test Parameters	1630

A.7.1.1.2.3	Test Requirements	1634

A.7.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1634

A.7.1.1.3.1	Test Purpose and Environment	1634

A.7.1.1.3.2	Test Parameters	1634

A.7.1.1.3.3	Test Requirements	1637

A.7.1.1.4	Cell reselection to FR2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1637

A.7.1.1.4.1	Test Purpose and Environment	1637

A.7.1.1.4.2	Test Parameters	1637

A.7.1.1.4.3	Test Requirements	1640

A.7.1.1.5	Cell reselection to FR2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1640

A.7.1.1.5.1	Test Purpose and Environment	1640

A.7.1.1.5.2	Test Parameters	1640

A.7.1.1.5.3	Test Requirements	1644

A.7.1.1.6	Cell reselection to FR2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1644

A.7.1.1.6.1	Test Purpose and Environment	1644

A.7.1.1.6.2	Test Parameters	1644

A.7.1.1.6.3	Test Requirements	1646

A.7.2	SA: RRC_INACTIVE state mobility	1647

A.7.3	RRC_CONNECTED state mobility	1647

A.7.3.1	Handover	1647

A.7.3.1.1	Inter-frequency handover from FR1 to FR2; unknown target cell	1647

A.7.3.1.1.1	Test Purpose and Environment	1647

A.7.3.1.1.2	Test Parameters	1647

A.7.3.1.1.3	Test Requirements	1651

A.7.3.1.2	Intra-frequency handover from FR2 to FR2; unknown target cell	1651

A.7.3.1.2.1	Test Purpose and Environment	1651

A.7.3.1.2.2	Test Parameters	1651

A.7.3.1.2.3	Test Requirements	1654

A.7.3.1.3	Inter-frequency handover from FR2 to FR2; unknown target cell	1654

A.7.3.1.3.1	Test Purpose and Environment	1654

A.7.3.1.3.2	Test Parameters	1654

A.7.3.1.3.3 Test Requirements	1655

A.7.3.1.4	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR2	1656

A.7.3.1.4.1	Test Purpose and Environment	1656

A.7.3.1.4.2	Test Parameters	1656

A.7.3.1.4.3 Test Requirements	1663

A.7.3.1.5	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR2	1663

A.7.3.1.5.1	Test Purpose and Environment	1663

A.7.3.1.5.2	Test Parameters	1663

A.7.3.1.5.3 Test Requirements	1670

A.7.3.2	RRC Connection Mobility Control	1670

A.7.3.2.1	SA: RRC Re-establishment	1670

A.7.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	1670

A.7.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	1673

A.7.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	1676

A.7.3.2.1.3.1	Test Purpose and Environment	1676

A.7.3.2.1.3.2	Test Requirements	1678

A.7.3.2.2	Random Access	1679

A.7.3.2.2.1	4-step RA type c ontention based random access test in FR2 for NR Standalone	1679

A.7.3.2.2.2	4-step RA type n on-contention based random access test in FR2 for NR Standalone	1683

A.7.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	1687

A.7.3.2.2.4	2-step RA type n on-contention based random access test in FR2 for NR Standalone	1690

A.7.3.2.3	SA: RRC Connection Release with Redirection	1693

A.7.3.2.3.1	Redirection from NR in FR2 to NR in FR2	1693

A.7.3.3 Conditional Handover	1696

A.7.3.3.1	Intra-frequency conditional handover from FR2 to FR2	1696

A.7.3.3.1.1	Test Purpose and Environment	1696

A.7.3.3.1.2	Test Parameters	1696

A.7.3.3.1.2.3	Test Requirements	1699

A.7.3.3.2	Inter-frequency handover from FR2 to FR2; unknown target cell	1699

A.7.3.3.2.1	Test Purpose and Environment	1699

A.7.3.3.2.2	Test Parameters	1699

A.7.3.3.2.3 Test Requirements	1701

A.7.4	Timing	1701

A.7.4.1	UE transmit timing	1701

A.7.4.1.1	NR UE Transmit Timing Test for FR2	1701

A.7.4.1.1.1	Test Purpose and environment	1701

A.7.4.1.1.2	Test requirements	1704

A.7.4.2	UE timer accuracy	1705

A.7.4.3	Timing advance	1705

A.7.4.3.1	SA FR2 timing advance adjustment accuracy	1705

A.7.4.3.1.1	Test Purpose and Environment	1705

A.7.4.3.1.2	Test Parameters	1705

A.7.4.3.1.3 Test Requirements	1708

A.7.5	Signaling characteristics	1709

A.7.5.1	Radio link Monitoring	1709

A.7.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	1709

A.7.5.1.1.1	Test Purpose and Environment	1709

A.7.5.1.1.2	Test Requirements	1712

A.7.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	1713

A.7.5.1.2.1	Test Purpose and Environment	1713

A.7.5.1.2.2	Test Requirements	1717

A.7.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	1718

A.7.5.1.3.1	Test Purpose and Environment	1718

A.7.5.1.3.2	Test Requirements	1721

A.7.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	1721

A.7.5.1.4.1	Test Purpose and Environment	1721

A.7.5.1.4.2	Test Requirements	1726

A.7.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	1726

A.7.5.1.5.1	Test Purpose and Environment	1726

A.7.5.1.5.2	Test Requirements	1730

A.7.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	1730

A.7.5.1.6.1	Test Purpose and Environment	1730

A.7.5.1.6.2	Test Requirements	1734

A.7.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	1734

A.7.5.1.7.1	Test Purpose and Environment	1734

A.7.5.1.7.2	Test Requirements	1737

A.7.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	1737

A.7.5.1.8.1	Test Purpose and Environment	1737

A.7.5.1.8.2	Test Requirements	1742

A.7.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	1742

A.7.5.1.9.1	Test Purpose and Environment	1742

A.7.5.1.9.2	Test Requirements	1745

A.7.5.2	Interruption	1745

A.7.5.2.1	Interruptions during measurements on deactivated NR SCC in FR2	1745

A.7.5.2.1.1	Test Purpose and Environment	1745

A.7.5.2.1.2	Test Requirements	1748

A.7.5.2.2	SA interruptions at NR SRS carrier-based switching	1749

A.7.5.2.2.1	Test Purpose and Environment	1749

A.7.5.2.2.2	Test Parameters	1749

A.7.5.2.2.3	Test Requirements	1751

A.7.5.3	SCell Activation and Deactivation Delay	1751

A.7.5.3.1	SCell Activation and deactivation for SCell in FR2 intra-band in non-DRX	1751

A.7.5.3.1.1	Test Purpose and Environment	1751

A.7.5.3.1.2	Test Requirements	1753

A.7.5.3.2	SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2	1753

A.7.5.3.2.1	Test Purpose and Environment	1753

A.7.5.3.2.2	Test Requirements	1758

A.7.5.3.3	SCell Activation and deactivation for SCell in FR2 inter-band in non-DRX	1759

A.7.5.3.3.1	Test Purpose and Environment	1759

A.7.5.3.3.2	Test Requirements	1762

A.7.5.3.4	Direct SCell activation at SCell addition of known SCell in FR2	1763

A.7.5.3.4.1	Test Purpose and Environment	1763

A.7.5.3.4.2	Test Requirements	1766

A.7.5.3.5	Direct SCell activation at handover with known SCell in FR2	1767

A.7.5.3.5.1	Test Purpose and Environment	1767

A.7.5.3.5.2	Test Requirements	1769

A.7.5.4	Void	1770

A.7.5.5	Beam Failure Detection and Link recovery procedures	1770

A.7.5.5.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	1770

A.7.5.5.1.1	Test Purpose and Environment	1770

A.7.5.5.1.2	Test Requirements	1775

A.7.5.5.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	1775

A.7.5.5.2.1	Test Purpose and Environment	1775

A.7.5.5.2.2	Test Requirements	1780

A.7.5.5.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	1780

A.7.5.5.3.1	Test Purpose and Environment	1780

A.7.5.5.3.2	Test Requirements	1786

A.7.5.5.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	1786

A.7.5.5.4.1	Test Purpose and Environment	1786

A.7.5.5.4.2	Test Requirements	1792

A.7.5.5.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	1792

A.7.5.5.5.1	Test Purpose and Environment	1792

A.7.5.5.5.2	Test Requirements	1797

A.7.5.5.6	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	1797

A.7.5.5.6.1	Test Purpose and Environment	1797

A.7.5.5.6.2	Test Requirements	1801

A.7.5.5.7	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	1801

A.7.5.5.7.1	Test Purpose and Environment	1801

A.7.5.5.7.2	Test Requirements	1805

A.7.5.6	Active BWP switch	1805

A.7.5.6.1	DCI-based and Timer-based Active BWP Switch	1805

A.7.5.6.1.1	NR FR2- NR FR2 DL active BWP switch of PCell with non-DRX in SA	1805

A.7.5.6.1.2	NR FR1- NR FR2 DL active BWP switch of PCell with non-DRX in SA	1810

A.7.5.6.1.3	NR FR2 DL active BWP switch with non-DRX in SA	1815

A.7.5.6.1.3.1	Test Purpose and Environment	1815

A.7.5.6.1.3.2	Test Requirements	1818

A.7.5.6.2	RRC-based Active BWP Switch	1818

A.7.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1822

A.7.5.6.3.1.1	Test Purpose and Environment	1822

A.7.5.6.3.1.2	Test Requirements	1825

A.7.5.6.4	SCell dormancy switch	1825

A.7.5.6.4.1	NR FR2 PCell SCell dormancy switch of single FR2 SCell inside active time	1825

A.7.5.6.4.1.1	Test Purpose and Environment	1825

A.7.5.6.4.1.2	Test Requirements	1830

A.7.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR2 SCells outside active time	1830

A.7.5.6.4.2.1	 Test Purpose and Environment	1830

A.7.5.6.4.2.2	 Test Requirements	1835

A.7.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1835

A.7.5.6.5.1	Active BWP switch on multiple SCells with non-DRX in SA	1835

A.7.5.7	PSCell addition and release delay	1838

A.7.5.7.1	Addition and Release Delay of known NR PSCell	1838

A.7.5.7.1.1	Test Purpose and Environment	1838

A.7.5.7.2	Addition and Release Delay of unknown NR PSCell	1842

A.7.5.7.2.1	Test Purpose and Environment	1842

A.7.5.7.2.2	Test Requirements	1846

A.7.5.8	Active TCI state switch delay	1846

A.7.5.8.1	MAC-CE based active TCI state switch	1846

A.7.5.8.2	RRC based active TCI state switch	1848

A.7.5.9	Uplink spatial relation switch delay	1851

A.7.5.9.1.1.1	Test Purpose and Environment	1851

A.7.5.9.1.1.2	Test Requirements	1854

A.7.5.9.2	RRC based spatial relation switch	1854

A.7.5.9.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	1854

A.7.5.9.2.1.1	Test Purpose and Environment	1854

A.7.5.9.2.1.2	Test Requirements	1857

A.7.5.10	UE specific CBW change	1857

A.7.5.10.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	1857

A.7.5.10.1.1	Test Purpose and Environment	1857

A.7.5.10.1.2	Test Requirements	1860

A.7.6	Measurement procedure	1861

A.7.6.1	Intra-frequency Measurements	1861

A.7.6.1.1	SA event triggered reporting test without gap under non-DRX	1861

A.7.6.1.1.1	Test purpose and Environment	1861

A.7.6.1.1.2	Test Requirements	1864

A.7.6.1.2	SA event triggered reporting test without gap under DRX	1864

A.7.6.1.2.1	Test purpose and Environment	1864

A.7.6.1.2.2	Test Requirements	1867

A.7.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	1868

A.7.6.1.3.1	Test purpose and Environment	1868

A.7.6.1.3.2	Test Requirements	1871

A.7.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	1871

A.7.6.1.4.1	Test purpose and Environment	1871

A.7.6.1.4.2	Test Requirements	1874

A.7.6.2	Inter-frequency Measurements	1875

A.7.6.2.1	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2)	1875

A.7.6.2.1.1	Test Purpose and Environment	1875

A.7.6.2.1.2	Test Requirements	1878

A.7.6.2.2	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is used (PCell in FR2)	1878

A.7.6.2.2.1	Test Purpose and Environment	1878

A.7.6.2.2.2	Test Requirements	1882

A.7.6.2.3	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is not used (PCell in FR2)	1882

A.7.6.2.3.1	Test Purpose and Environment	1882

A.7.6.2.3.2	Test Requirements	1886

A.7.6.2.4	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is used (PCell in FR2)	1886

A.7.6.2.4.1	Test Purpose and Environment	1886

A.7.6.2.4.2	Test Requirements	1890

A.7.6.2.5	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1)	1890

A.7.6.2.5.1	Test Purpose and Environment	1890

A.7.6.2.5.2	Test Requirements	1894

A.7.6.2.6	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (PCell in FR1)	1894

A.7.6.2.6.1	Test Purpose and Environment	1894

A.7.6.2.6.2	Test Requirements	1899

A.7.6.2.7	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1)	1900

A.7.6.2.7.1	Test Purpose and Environment	1900

A.7.6.2.7.2	Test Requirements	1904

A.7.6.2.8	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1)	1904

A.7.6.2.8.1	Test Purpose and Environment	1904

A.7.6.2.8.2	Test Requirements	1909

A.7.6.2.9	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2) (rel16 additional mandatory gap pattern 17)	1910

A.7.6.2.9.1	Test Purpose and Environment	1910

A.7.6.2.9.2	Test Requirements	1914

A.7.6.2.10	SA event triggered reporting test without gap under non-DRX	1914

A.7.6.2.10.1	Test Purpose and Environment	1914

A.7.6.2.10.2	Test Requirements	1916

A.7.6.2.11	SA event triggered reporting test without gap under DRX	1916

A.7.6.2.11.1	Test Purpose and Environment	1916

A.7.6.2.11.2	Test Requirements	1919

A.7.6.3	L1-RSRP measurement for beam reporting	1920

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1920

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1920

A.7.6.3.1.1	Test Purpose and Environment	1920

A.7.6.3.1.2	Test parameters	1920

A.7.6.3.1.3	Test Requirements	1922

A.7.6.3.2	SSB based L1-RSRP measurement when DRX is used	1922

A.7.6.3.2.1	Test Purpose and Environment	1922

A.7.6.3.2.2	Test parameters	1923

A.7.6.3.2.3	Test Requirements	1925

A.7.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	1925

A.7.6.3.3.1	Test Purpose and Environment	1925

A.7.6.3.3.2	Test parameters	1926

A.7.6.3.3.3	Test Requirements	1928

A.7.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	1929

A.7.6.3.4.1	Test Purpose and Environment	1929

A.7.6.3.4.2	Test parameters	1929

A.7.6.3.3.3	Test Requirements	1931

A.7.6.4	CLI measurements	1932

A.7.6.4.1	SRS-RSRP measurement with non-DRX	1932

A.7.6.4.1.1	Test Purpose and Environment	1932

A.7.6.4.1.2	Test Parameters	1932

A.7.6.4.1.3	Test Requirements	1934

A.7.6.4.2	CLI-RSSI measurement with non-DRX	1934

A.7.6.4.2.1	Test Purpose and Environment	1934

A.7.6.4.2.2	Test Parameters	1935

A.7.6.4.2.3	Test Requirements	1936

A.7.6.5.1   SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2)	1937

A.7.6.5.1.1	Test Purpose and Environment	1937

A.7.6.5.1.2	Test Requirements	1940

A.7.6.6	L1-SINR measurement for beam reporting	1940

A.7.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	1942

A.7.6.6.2.1	Test Purpose and Environment	1942

A.7.6.6.2.2	Test parameters	1943

A.7.6.6.2.3	Test Requirements	1945

A.7.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	1946

A.7.6.6.3.1	Test Purpose and Environment	1946

A.7.6.6.3.2	Test parameters	1946

A.7.6.6.3.3	Test Requirements	1948

A.7.6.7	CSI-RS based intra-frequency Measurements	1948

A.7.6.7.1	SA event triggered reporting test without gap under DRX for CSI-RS based intra-frequency measurement	1948

A.7.6.7.1.1	Test purpose and Environment	1948

A.7.6.7.1.2	Test Requirements	1951

A.7.6.8	CSI-RS based inter-frequency Measurements	1951

A.7.6.8.1	SA event triggered reporting tests for FR2 CSI-RS based measurement when non-DRX is used (PCell in FR2)	1951

A.7.6.8.1.1	Test Purpose and Environment	1951

A.7.6.8.1.2	Test Requirements	1954

A.7.6.9	RSTD measurements	1954

A.7.6.9.1	 NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA	1954

A.7.6.9.1.1	Test Purpose and Environment	1954

A.7.6.9.1.2	Test Requirements	1960

A.7.6.9.2	 NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR2 SA	1960

A.7.6.9.2.1	Test Purpose and Environment	1960

A.7.6.9.2.2	Test Requirements	1967

A.7.6.10 PRS-RSRP measurements	1967

A.7.6.10.1 PRS-RSRP reporting delay test case for single positioning frequency layer	1967

A.7.6.10.1.1	Test Purpose and Environment	1967

A.7.6.10.1.2	Test Requirements	1971

A.7.6.10.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	1971

A.7.6.10.2.1	Test Purpose and Environment	1971

A.7.6.10.2.2	Test Requirements	1975

A.7.6.11	UE Rx-Tx time difference measurements	1975

A.7.6.11.1	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	1975

A.7.6.11.1.1	Test purpose and environment	1975

A.7.6.11.1.2	Test requirements	1978

A.7.6.11.2	UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR2 SA	1978

A.7.6.11.2.1	Test purpose and environment	1978

A.7.6.11.2.2	Test requirements	1981

A.7.7	Measurement Performance requirements	1981

A.7.7.1	SS-RSRP	1981

A.7.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1981

A.7.7.1.1.1	Test Purpose and Environment	1981

A.7.7.1.1.2	Test parameters	1981

A.7.7.1.1.3	Test Requirements	1986

A.7.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1987

A.7.7.1.2.1	Test Purpose and Environment	1987

A.7.7.1.2.2	Test parameters	1987

A.7.7.1.2.3	Test Requirements	1991

A.7.7.1.3	SA inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	1992

A.7.7.1.3.1	Test Purpose and Environment	1992

A.7.7.1.3.2	Test parameters	1992

A.7.7.1.3.3	Test Requirements	1995

A.7.7.2	SS-RSRQ	1996

A.7.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	1996

A.7.7.2.1.1	Test Purpose and Environment	1996

A.7.7.2.1.2	Test Parameters	1996

A.7.7.2.1.3	Test Requirements	1998

A.7.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1998

A.7.7.2.2.1	Test Purpose and Environment	1998

A.7.7.2.2.2	Test Parameters	1998

A.7.7.2.2.3	Test Requirements	2000

A.7.7.3	SS-SINR	2000

A.7.7.3.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2000

A.7.7.3.1.1	Test Purpose and Environment	2000

A.7.7.3.1.2	Test Parameters	2000

A.7.7.3.1.3	Test Requirements	2003

A.7.7.3.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2003

A.7.7.3.2.1	Test Purpose and Environment	2003

A.7.7.3.2.2	Test Parameters	2003

A.7.7.3.2.3	Test Requirements	2005

A.7.7.4	L1-RSRP measurement for beam reporting	2005

A.7.7.4.1	SSB based L1-RSRP measurement	2005

A.7.7.4.1.1	Test Purpose and Environment	2005

A.7.7.4.1.2	Test parameters	2006

A.7.7.4.1.3	Test Requirements	2008

A.7.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	2009

A.7.7.4.2.1	Test Purpose and Environment	2009

A.7.7.4.2.2	Test parameters	2009

A.7.7.4.2.3	Test Requirements	2011

A.7.7.5	CLI measurements	2012

A.7.7.5.1	SA SRS-RSRP measurement accuracy with FR2 serving cell	2012

A.7.7.5.1.1	Test Purpose and Environment	2012

A.7.7.5.1.2	Test parameters	2012

A.7.7.5.1.3	Test Requirements	2015

A.7.7.5.2	SA CLI-RSSI measurement accuracy with FR2 serving cell	2016

A.7.7.5.2.1	Test Purpose and Environment	2016

A.7.7.5.2.2	Test parameters	2016

A.7.7.5.2.3	Test Requirements	2018

A.7.7.6	L1-SINR measurement for beam reporting	2019

A.7.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	2021

A.7.7.6.2.1	Test Purpose and Environment	2021

A.7.7.6.2.2	Test parameters	2022

A.7.7.6.2.3	Test Requirements	2024

A.7.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	2024

A.7.7.6.3.1	Test Purpose and Environment	2024

A.7.7.6.3.2	Test parameters	2025

A.7.7.6.3.3	Test Requirements	2027

A.7.7.7	CSI-RSRP	2027

A.7.7.7.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2027

A.7.7.7.1.1	Test Purpose and Environment	2027

A.7.7.7.1.2	Test parameters	2028

A.7.7.7.1.3	Test Requirements	2031

A.7.7.7.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2032

A.7.7.7.2.1	Test Purpose and Environment	2032

A.7.7.7.2.2	Test parameters	2032

A.7.7.7.2.3	Test Requirements	2035

A.7.7.8	CSI-RSRQ	2035

A.7.7.8.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	2035

A.7.7.8.1.1	Test Purpose and Environment	2035

A.7.7.8.1.2	Test Parameters	2036

A.7.7.8.1.3	Test Requirements	2038

A.7.7.8.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2038

A.7.7.8.2.1	Test Purpose and Environment	2038

A.7.7.8.2.2	Test Parameters	2038

A.7.7.8.2.3	Test Requirements	2040

A.7.7.9	CSI-SINR	2040

A.7.7.9.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2040

A.7.7.9.1.1	Test Purpose and Environment	2040

A.7.7.9.1.2	Test Parameters	2040

A.7.7.9.1.3	Test Requirements	2042

A.7.7.9.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2042

A.7.7.9.2.1	Test Purpose and Environment	2042

A.7.7.9.2.2	Test Parameters	2043

A.7.7.9.2.3	Test Requirements	2045

A.7.7.10	RSTD measurements	2045

A.7.7.10.1	RSTD measurement accuracy test case for single positioning frequency layer	2045

A.7.7.10.1.1	Test purpose and Environment	2045

A.7.7.10.1.2	Test Requirements	2047

A.7.7.10.2	RSTD measurement accuracy test case for dual positioning frequency layer	2047

A.7.7.10.2.1	Test purpose and Environment	2047

A.7.7.10.2.2	Test Requirements	2049

A.7.7.11	PRS-RSRP measurements	2049

A.7.7.11.1	SA measurement accuracy with PRS in FR2	2049

A.7.7.11.1.1	Test Purpose and Environment	2049

A.7.7.11.1.2	Test parameters	2049

A.7.7.11.1.3	Test Requirements	2052

A.7.7.12	UE Rx-Tx time difference measurements	2053

A.7.7.12.1	UE Rx-Tx time difference measurement period for single positioning frequency layer in FR2 SA	2053

A.7.7.12.1.1	Test purpose and environment	2053

A.7.7.12.1.2	Test parameters	2053

A.7.7.12.1.3	Test requirements	2055

A.8	E-UTRA standalone tests for NR RRM	2055

A.8.1	Void	2055

A.8.2	RRC_IDLE state mobility	2055

A.8.2.1	Inter-RAT NR Cell re-selection	2055

A.8.2.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	2055

A.8.2.1.1.1	Test Purpose and Environment	2055

A.8.2.1.1.2	Test Requirements	2060

A.8.2.1.2	E-UTRA Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16	2061

A.8.2.1.2.1	Test Purpose and Environment	2061

A.8.2.1.2.2	Test Requirements	2064

A.8.2.2	E-UTRA – NR Inter-RAT Early Measruement Reporting	2065

A.8.2.2.1	E-UTRA – NR Early Measruement Reporting for NR in FR1	2065

A.8.2.2.1.1	Test Purpose and Environment	2065

A.8.2.2.1.2	Test Requirements	2068

A.8.3	RRC_CONNECTED state mobility	2069

A.8.3.1	Handover	2069

A.8.3.1.1	E-UTRAN - NR handover in FR1	2069

A.8.3.1.1.1	Test Purpose and Environment	2069

A.8.3.1.1.2	Test Requirements	2074

A.8.4	Measurement procedure	2074

A.8.4.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay	2074

A.8.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay in non-DRX	2074

A.8.4.1.1.1	Test Purpose and Environment	2074

A.8.4.1.1.2	Test Requirements	2076

A.8.4.1.2	E-UTRA – NR Inter-RAT SFTD Measurement Delay in DRX	2077

A.8.4.1.2.1	Test Purpose and Environment	2077

A.8.4.1.2.2	Test Requirements	2078

A.8.4.2	E-UTRA – NR Inter-RAT Measurements	2078

A.8.4.2.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2078

A.8.4.2.1.1	Test Purpose and Environment	2078

A.8.4.2.1.2	Test Requirements	2083

A.8.4.2.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2083

A.8.4.2.2.1	Test Purpose and Environment	2083

A.8.4.2.2.2	Test Requirements	2087

A.8.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2087

A.8.4.2.3.1	Test Purpose and Environment	2087

A.8.4.2.3.2	Test Requirements	2092

A.8.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2092

A.8.4.2.4.1	Test Purpose and Environment	2092

A.8.4.2.4.2	Test Requirements	2096

A.8.4.2.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	2096

A.8.4.2.5.1	Test Purpose and Environment	2096

A.8.4.2.5.2	Test Requirements	2098

A.8.4.2.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	2099

A.8.4.2.6.1	Test Purpose and Environment	2099

A.8.4.2.6.2	Test Requirements	2101

A.8.4.2.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	2102

A.8.4.2.7.1	Test Purpose and Environment	2102

A.8.4.2.7.2	Test Requirements	2104

A.8.4.2.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	2105

A.8.4.2.8.1	Test Purpose and Environment	2105

A.8.4.2.8.2	Test Requirements	2107

A.8.4.2.9	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16	2108

A.8.4.2.9.1	Test Purpose and Environment	2108

A.8.4.2.9.2	Test Requirements	2112

A.8.5	Measurement performance	2112

A.8.5.1	SFTD accuracy	2112

A.8.5.1.1	SFTD accuracy	2112

A.8.5.1.1.1	Test Purpose	2112

A.8.5.1.1.2	Test Environment	2113

A.8.5.1.1.3	Test Requirements	2119

A.8.5.2	E-UTRA – NR Inter-RAT Measurement Performance requirements	2119

A.8.5.2.1.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	2119

A.8.5.2.1.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	2124

A.8.5.2.1.2.1	Test Purpose and Environment	2124

A.8.5.2.1.2.2	Test Parameters	2124

A.8.5.2.1.2.3	Test Requirements	2126

A.8.5.2.2	SS-RSRQ	2126

A.8.5.2.2.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	2126

A.8.5.2.2.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	2131

A.8.5.2.2.2.1	Test Purpose and Environment	2131

A.8.5.2.2.2.2	Test Parameters	2131

A.8.5.2.2.2.3	Test Requirements	2133

A.8.5.2.3	SS-SINR	2133

A.8.5.2.3.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	2133

A.8.5.2.3.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	2137

A.8.5.2.3.2.1	Test Purpose and Environment	2137

A.8.5.2.3.2.2	Test Parameters	2137

A.8.5.2.3.2.3	Test Requirements	2139

A.9	V2X Tests	2140

A.9.1	V2X Tests in FR1	2140

A.9.1.1	Test for V2X UE Transmit Timing	2140

A.9.1.1.1 Test for GNSS as Synchronization Reference Source	2140

A.9.1.1.1.1	Test Purpose and Environment	2140

A.9.1.1.1.2	Test requirements	2140

A.9.1.1.2	Test for SyncRef UE as Synchronization Reference Source	2140

A.9.1.1.2.1	Test Purpose and Environment	2140

A.9.1.1.2.2	Test requirements	2141

A.9.1.1.3	Test for FR1 NR Cell as Synchronization Reference Source	2141

A.9.1.1.3.1	Test Purpose and Environment	2141

A.9.1.1.3.2	Test requirements	2144

A.9.1.2	Test for Initiation/Cease of SLSS Transmission with V2X Sidelink Communication	2144

A.9.1.2.1	Test for FR1 NR Cell as synchronization reference source without gap under non-DRX	2144

A.9.1.2.1.1	Test Purpose and Environment	2144

A.9.1.2.1.2	Test Requirements	2147

A.9.1.2.2	Test for SyncRef UE as synchronization reference source	2147

A.9.1.2.2.1	Test Purpose and Environment	2147

A.9.1.2.2.2	Test Requirements	2148

A.9.1.3	 Test for V2X Synchronization Reference Selection/Reselection	2149

A.9.1.3.1	 Test for GNSS configured as the highest priority	2149

A.9.1.3.1.1	Test Purpose and Environment	2149

A.9.1.3.1.2	Test Requirements	2151

A.9.1.3.2	 Test for FR1 NR Cell configured as the highest priority	2152

A.9.1.3.2.1	Test Purpose and Environment	2152

A.9.1.3.2.2	Test Requirements	2154

A.9.1.4	Test for L1 SL-RSRP Measurement	2155

A.9.1.4.1	Test for V2X UE Autonomous Resource Selection/Reselection	2155

A.9.1.4.1.1	Test Purpose and Environment	2155

A.9.1.4.1.2	Test Requirements	2157

A. 9.1.4.2 Test for V2X UE Resource Pre-emption	2157

A. 9.1.4.2.1	Test Purpose and Environment	2157

A.9.1.4.2.2	Test Requirements	2161

A.9.1.4.3	 Test for V2X UE Resource Re-evaluation	2161

A.9.1.4.3.1	Test Purpose and Environment	2161

A.9.1.4.3.2	Test Requirements	2165

A.9.1.5	Test for Congestion Control Measurement	2165

A.9.1.5.1	Test Purpose and Environment	2165

A.9.1.5.2	Test Requirements	2168

A.9.1.6	Test for Interruption	2168

A.9.1.6.1	Test for Interruption to WAN due to V2X Sidelink Communication	2168

A.9.1.6.1.1	Test Purpose and Environment	2168

A.9.1.6.1.2	Test Requirements	2172

A.10	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	2172

A.10.1	RRC_CONNECTED state mobility	2172

A.10.1.1	RRC connection mobility control	2172

A.10.1.1.1	Random Access	2172

A.10.1.1.1.1	4-step RA type contention-based random access for NR PSCell with CCA	2172

A.10.1.1.1.1.1	Test Purpose and Environment	2172

A.10.1.1.1.1.2	Test Requirements	2175

A.10.1.1.1.1.2.1	Random Access Preamble Transmission	2175

A.10.1.1.1.1.2.2	Random Access Response Reception	2175

A.10.1.1.1.1.2.3	No Random Access Response Reception	2176

A.10.1.1.1.1.2.4	Receiving an UL grant for msg3 retransmission	2176

A.10.1.1.1.1.2.5	 Contention Resolution Timer expiry	2176

A.10.1.1.1.2	4-step RA type non-contention based random access for NR PSCell with CCA	2177

A.10.1.1.1.2.1	Test Purpose and Environment	2177

A.10.1.1.1.2.2	Test Requirements	2180

A.10.1.1.1.2.2.1	SSB-based Random Access Preamble Transmission	2180

A.10.1.1.1.2.2.2	Random Access Response Reception	2181

A.10.1.1.1.2.2.3	No Random Access Response Reception	2181

A.10.1.1.1.3	2-step RA type contention-based random access for NR PSCell with CCA	2181

A.10.1.1.1.3.1	Test Purpose and Environment	2181

A.10.1.1.1.3.2	Test Requirements	2185

A.10.1.1.1.3.2.1	MsgA Transmission	2185

A.10.1.1.1.3.2.2	MsgB Reception	2185

A.10.1.1.1.3.2.3	No MsgB Reception	2186

A.10.1.1.1.4	2-step RA type non-contention based random access for NR PSCell with CCA	2186

A.10.1.1.1.4.1	Test Purpose and Environment	2186

A.10.1.1.1.4.2	Test Requirements	2190

A.10.1.1.1.4.2.1	MsgA Transmission	2190

A.10.1.1.1.4.2.2	MsgB Reception	2191

A.10.1.1.1.4.2.3	No MsgB Reception	2191

A.10.2	Timing	2192

A.10.2.1	UE transmit timing	2192

A.10.2.1.1	UE Transmit Timing Test with PSCell under DL CCA	2192

A.10.2.1.1.1	Test Purpose and environment	2192

A.10.2.1.1.2	Test requirements	2195

A.10.2.2	UE timing advance	2196

A.10.2.2.1	UE Timing Advance Adjustment Accuracy with PSCell under DL CCA	2196

A.10.2.2.1.1	Test Purpose and Environment	2196

A.10.2.2.1.2	Test Parameters	2196

A.10.2.2.1.3	Test Requirements	2199

A.10.3	Signalling characteristics	2199

A.10.3.1	Radio link monitoring	2199

A.10.3.1.1	Introduction	2199

A.10.3.1.2	Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	2200

A.10.3.1.2.1	Test purpose and environment	2200

A.10.3.1.2.2	Test requirements	2204

A.10.3.1.3	Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	2204

A.10.3.1.3.1	Test purpose and environment	2204

A.10.3.1.3.2	Test requirements	2210

A.10.3.1.4	Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode	2210

A.10.3.1.4.1	Test purpose and environment	2210

A.10.3.1.4.2	Test requirements	2210

A.10.3.1.5	Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode	2210

A.10.3.1.5.1	Test purpose and environment	2210

A.10.3.1.5.2	Test requirements	2210

A.10.3.2	Interruption	2210

A.10.3.2.1	E-UTRAN – NR interruptions during SCell operations with CCA	2210

A.10.3.2.1.1	Test Purpose and Environment	2210

A.10.3.2.1.2	Test Requirements	2215

A.10.3.3	SCell activation and deactivation delay	2215

A.10.3.3.1	SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 160 ms SCell measurement cycle	2215

A.10.3.3.1.1	Test Purpose and Environment	2215

A.10.3.3.1.2	Test Requirements	2220

A.10.3.3.2 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 320 ms SCell measurement cycle	2220

A.10.3.3.2.1	Test Purpose and Environment	2220

A.10.3.3.2.2	Test Requirements	2221

A.10.3.3.3	SCell Activation and Deactivation of unknown NR SCell with NR PSCell and NR SCell under CCA	2221

A.10.3.3.3.1	Test Purpose and Environment	2221

A.10.3.3.3.2	Test Requirements	2222

A.10.3.4	Beam failure detection and link recovery procedures	2222

A.10.3.4.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	2222

A.10.3.4.1.1	Test Purpose and Environment	2222

A.10.3.4.1.2	Test Requirements	2228

A.10.3.4.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	2228

A.10.3.4.2.1	Test Purpose and Environment	2228

A.10.3.4.2.2	Test Requirements	2235

A.10.3.5	Active BWP switching	2235

A.10.3.5.1	UL active BWP switch delay with consistent UL LBT failure on PSCell subject to UL CCA in EN-DC	2235

A.10.3.5.1.2	Test Requirements	2240

A.10.3.5.2	DCI-based and Timer-based Active BWP Switch	2241

A.10.3.5.2.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	2241

A.10.3.5.2.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	2244

A.10.3.5.3	RRC-based Active BWP Switch	2248

A.10.3.5.3.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	2248

A.10.3.6	PSCell addition and release delay	2251

A.10.3.6.1	Addition and Release Delay of known NR PSCell on the carrier under CCA	2251

A.10.3.6.1.1	Test purpose and environment	2251

A.10.3.6.1.2	Test Requirements	2255

A.10.3.7	Active TCI state switching delay	2256

A.10.4	Measurement procedure	2256

A.10.4.1	Intra-frequency measurements	2256

A.10.4.1.1	Event-triggered reporting tests on PSCC without gaps under non-DRX	2256

A.10.4.1.1.1	Test purpose and environment	2256

A.10.4.1.1.2	Test parameters	2256

A.10.4.1.1.3	Test Requirements	2256

A.10.4.1.2	Event-triggered reporting tests on PSCC without gaps under DRX	2257

A.10.4.1.2.1	Test purpose and environment	2257

A.10.4.1.2.2	Test parameters	2257

A.10.4.1.2.3	Test Requirements	2257

A.10.4.1.3	Event-triggered reporting tests on PSCC with per-UE gaps under non-DRX	2257

A.10.4.1.3.1	Test purpose and environment	2257

A.10.4.1.3.2	Test parameters	2257

A.10.4.1.3.3	Test Requirements	2257

A.10.4.1.4	Event-triggered reporting tests on PSCC with per-UE gaps under DRX	2258

A.10.4.1.4.1	Test purpose and environment	2258

A.10.4.1.4.2	Test parameters	2258

A.10.4.1.4.3	Test Requirements	2258

A.10.4.1.5	Event-triggered reporting tests on SCC without gaps under non-DRX	2258

A.10.4.1.5.1	Test purpose and environment	2258

A.10.4.1.5.2	Test parameters	2258

A.10.4.1.5.3	Test Requirements	2258

A.10.4.1.6	Event-triggered reporting tests on SCC without gaps under DRX	2259

A.10.4.1.6.1	Test purpose and environment	2259

A.10.4.1.6.2	Test parameters	2259

A.10.4.1.6.3	Test Requirements	2259

A.10.4.1.7	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	2259

A.10.4.1.7.1	Test purpose and environment	2259

A.10.4.1.7.2	Test parameters	2259

A.10.4.1.7.3	Test Requirements	2259

A.10.4.1.8	Event-triggered reporting tests on SCC with per-UE gaps under DRX	2260

A.10.4.1.8.1	Test purpose and environment	2260

A.10.4.1.8.2	Test parameters	2260

A.10.4.1.8.3	Test Requirements	2260

A.10.4.1.9	RSSI measurement reporting on PSCC	2260

A.10.4.1.9.1	Test purpose and environment	2260

A.10.4.1.9.2	Test parameters	2260

A.10.4.1.10	Channel occupancy measurement reporting on PSCC	2261

A.10.4.1.10.1	Test purpose and environment	2261

A.10.4.1.10.2	Test parameters	2261

A.10.4.1.11	RSSI measurement reporting on SCC	2261

A.10.4.1.11.1	Test purpose and environment	2261

A.10.4.1.11.2	Test parameters	2261

A.10.4.1.12	Channel occupancy measurement reporting on SCC	2262

A.10.4.1.12.1	Test purpose and environment	2262

A.10.4.1.12.2	Test parameters	2262

A.10.4.2	Inter-frequency measurements	2262

A.10.4.2.1	RSSI measurement reporting	2262

A.10.4.2.1.1	Test purpose and environment	2262

A.10.4.2.1.2	Test parameters	2262

A.10.4.2.2	Channel occupancy measurement reporting	2263

A.10.4.2.2.1	Test purpose and environment	2263

A.10.4.2.2.2	Test parameters	2263

A.10.4.2.3	EN-DC event triggered reporting tests for FR1 with CCA cell without SSB time index detection when DRX is not used	2263

A.10.4.2.3.1	Test Purpose and Environment	2263

A.10.4.2.3.2	Test Requirements	2268

A.10.4.2.4	EN-DC event triggered reporting tests for FR1 cell with CCA without SSB time index detection when DRX is used	2269

A.10.4.2.4.1	Test Purpose and Environment	2269

A.10.4.2.4.2	Test Requirements	2273

A.10.4.2.5	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is not used	2274

A.10.4.2.5.1	Test Purpose and Environment	2274

A.10.4.2.5.2	Test Requirements	2278

A.10.4.2.6	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is used	2279

A.10.4.2.6.1	Test Purpose and Environment	2279

A.10.4.2.6.2	Test Requirements	2283

A.10.4.2.7	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	2284

A.10.4.2.7.1	Test Purpose and Environment	2284

A.10.4.2.7.2	Test Requirements	2290

A.10.4.2.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	2290

A.10.4.2.8.1	Test Purpose and Environment	2290

A.10.4.2.8.2	Test Requirements	2296

A.10.4.2.9	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	2297

A.10.4.2.9.1	Test Purpose and Environment	2297

A.10.4.2.9.2	Test Requirements	2302

A.10.4.2.10	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	2302

A.10.4.2.10.1	Test Purpose and Environment	2302

A.10.4.2.10.2	Test Requirements	2308

A.10.4.3	L1-RSRP measurements for beam reporting	2309

A.10.4.3.1	SSB based L1-RSRP measurement on PSCC when DRX is not used	2309

A.10.4.3.1.1	Test Purpose and Environment	2309

A.10.4.3.1.2	Test parameters	2309

A.10.4.3.1.3	Test Requirements	2311

A.10.4.3.2	SSB based L1-RSRP measurement on PSCC when DRX is used	2312

A.10.4.3.2.1	Test Purpose and Environment	2312

A.10.4.3.2.2	Test parameters	2312

A.10.4.3.2.3	Test Requirements	2314

A.10.4.3.3	SSB based L1-RSRP measurement on SCC when DRX is not used	2315

A.10.4.3.3.1	Test Purpose and Environment	2315

A.10.4.3.3.2	Test parameters	2315

A.10.4.3.3.3	Test Requirements	2318

A.10.4.3.4	SSB based L1-RSRP measurement on SCC when DRX is used	2319

A.10.4.3.4.1	Test Purpose and Environment	2319

A.10.4.3.4.2	Test parameters	2319

A.10.4.3.4.3	Test Requirements	2322

A.10.4.4	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	2323

A.10.4.4.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2323

A.10.4.4.1.1	Test Purpose and Environment	2323

A.10.4.4.1.2	Test Requirements	2327

A.10.4.4.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2328

A.10.4.4.2.1	Test Purpose and Environment	2328

A.10.4.4.2.2	Test Requirements	2333

A.10.4.4.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2334

A.10.4.4.3.1	Test Purpose and Environment	2334

A.10.4.4.3.2	Test Requirements	2338

A.10.4.4.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2339

A.10.4.4.4.1	Test Purpose and Environment	2339

A.10.4.4.4.2	Test Requirements	2344

A.10.5	Measurement performance	2345

A.10.5.1	SS-RSRP	2345

A.10.5.1.1	Intra-frequency measurement accuracy on a CCA serving cell	2345

A.10.5.1.1.1	Test Purpose and Environment	2345

A.10.5.1.1.2	Test parameters	2345

A.10.5.1.1.3	Test Requirements	2348

A.10.5.1.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	2348

A.10.5.1.2.1	Test Purpose and Environment	2348

A.10.5.1.2.2	Test parameters	2348

A.10.5.1.2.3	Test Requirements	2351

A.10.5.2	SS-RSRQ	2351

A.10.5.2.1	Intra-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	2351

A.10.5.2.1.1	Test Purpose and Environment	2351

A.10.5.2.1.2	Test Parameters	2351

A.10.5.2.1.3	Test Requirements	2354

A.10.5.2.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	2354

A.10.5.2.2.1	Test Purpose and Environment	2354

A.10.5.2.2.2	Test Parameters	2354

A.10.5.2.2.3	Test Requirements	2357

A.10.5.3	SS-SINR	2357

A.10.5.4	L1-RSRP measurement for beam reporting with CCA serving cell	2357

A.10.5.4.1	SSB based L1-RSRP measurement	2357

A.10.5.4.1.1	Test Purpose and Environment	2357

A.10.5.4.1.2	Test parameters	2357

A.10.5.4.1.3	Test Requirements	2360

A.10.5.5	RSSI	2360

A.10.5.5.1 	RSSI measurement accuracy on PSCC with CCA	2360

A.10.5.5.1.1	Test Purpose and Environment	2360

A.10.5.5.1.2	Test parameters	2360

A.10.5.5.1.3	Test Requirements	2363

A.10.5.5.2 	RSSI measurement accuracy on SCC with CCA	2363

A.10.5.5.2.1	Test Purpose and Environment	2363

A.10.5.5.2.2	Test parameters	2363

A.10.5.5.2.3	Test Requirements	2366

A.10.5.5.3 	Inter-frequency RSSI measurement accuracy on a carrier with CCA	2366

A.10.5.5.3.1	Test Purpose and Environment	2366

A.10.5.5.3.2	Test parameters	2366

A.10.5.5.3.3	Test Requirements	2369

A.10.5.6	Channel occupancy	2369

A.10.5.6.1 	Channel occupancy measurement accuracy on PSCC with CCA	2369

A.10.5.6.1.1	Test Purpose and Environment	2369

A.10.5.6.1.2	Test parameters	2369

A.10.5.6.1.3	Test Requirements	2373

A.10.5.6.2 	Channel occupancy measurement accuracy on SCC with CCA	2373

A.10.5.6.2.1	Test Purpose and Environment	2373

A.10.5.6.2.2	Test parameters	2373

A.10.5.6.2.3	Test Requirements	2376

A.10.5.6.3 	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	2376

A.10.5.6.3.1	Test Purpose and Environment	2376

A.10.5.6.3.2	Test parameters	2376

A.10.5.6.3.3	Test Requirements	2380

A.11	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	2380

A.11.1	RRC_IDLE state mobility	2380

A.11.1.1	Cell re-selection with both source and target NR carrier frequencies under CCA	2380

A.11.1.1.1	Cell reselection to FR1 intra-frequency NR cells when subject to CCA on the serving and target cell	2380

A.11.1.1.1.1	Test Purpose and Environment	2380

A.11.1.1.1.2	Test Parameters	2380

A.11.1.1.1.3	Test Requirements	2383

A.11.1.1.2	Cell reselection to FR1 inter-frequency NR case when subject to CCA on the serving and target cell	2383

A.11.1.1.2.1	Test Purpose and Environment	2383

A.11.1.1.2.2	Test Parameters	2383

A.11.1.1.2.3	Test Requirements	2387

A.11.1.2	Cell re-selection to NR with source NR carrier frequency under CCA	2387

A.11.1.2.1	Cell reselection to FR1 inter-frequency NR case when serving cell is subject to CCA	2387

A.11.1.2.1.1	Test Purpose and Environment	2387

A.11.1.2.1.2	Test Parameters	2388

A.11.1.2.1.3	Test Requirements	2394

A.11.1.3	Cell re-selection from NR carrier with target NR carrier frequency under CCA	2395

A.11.1.3.1	Cell reselection to FR1 inter-frequency NR case when target cell is subject to CCA	2395

A.11.1.3.1.1	Test Purpose and Environment	2395

A.11.1.3.1.2	Test Parameters	2395

A.11.1.3.1.3	Test Requirements	2401

A.11.1.4	Inter-RAT cell re-selection to E-UTRAN with source NR carrier frequency under CCA	2402

A.11.1.4.1	Cell reselection to higher priority E-UTRAN when serving cell is subject to CCA	2402

A.11.1.4.1.1	Test Purpose and Environment	2402

A.11.1.4.1.2	Test Parameters	2402

A.11.1.4.1.3	Test Requirements	2405

A.11.1.4.2	Cell reselection to lower priority E-UTRAN when serving cell is subject to CCA	2406

A.11.1.4.2.1	Test Purpose and Environment	2406

A.11.1.4.2.2	Test Requirements	2408

A.11.2	RRC_CONNECTED state mobility	2409

A.11.2.1	Handover	2409

A.11.2.1.1	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; known target cell	2409

A.11.2.1.1.1	Test Purpose and Environment	2409

A.11.2.1.1.2	Test Parameters	2409

A.11.2.1.1.3 Test Requirements	2412

A.11.2.1.2	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	2412

A.11.2.1.2.1	Test Purpose and Environment	2412

A.11.2.1.2.2	Test Parameters	2412

A.11.2.1.2.3	Test Requirements	2415

A.11.2.1.3	Inter-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	2415

A.11.2.1.3.1	Test Purpose and Environment	2415

A.11.2.1.3.2	Test Parameters	2415

A.11.2.1.3.3 Test Requirements	2418

A.11.2.1.4	Inter-frequency handover from FR1 carrier under CCA to FR1; known target cell	2418

A.11.2.1.4.1	Test Purpose and Environment	2418

A.11.2.1.4.2	Test Parameters	2418

A.11.2.1.4.3 Test Requirements	2422

A.11.2.1.5	Inter-frequency handover from FR1 carrier under CCA to FR1; unknown target cell	2422

A.11.2.1.5.1	Test Purpose and Environment	2422

A.11.2.1.5.2	Test Parameters	2422

A.11.2.1.5.3 Test Requirements	2426

A.11.2.1.6	Inter-frequency handover from FR1 to FR1 carrier under CCA; unknown target cell	2427

A.11.2.1.6.1	Test Purpose and Environment	2427

A.11.2.1.6.2	Test Parameters	2427

A.11.2.1.6.3 Test Requirements	2430

A.11.2.1.7	 SA NR FR1 carrier under CCA - E-UTRAN handover with known target cell	2431

A.11.2.1.7.1	Test Purpose and Environment	2431

A.11.2.1.7.2	Test Requirements	2437

A.11.2.1.8	SA NR FR1 carrier under CCA - E-UTRAN handover with unknown target cell	2437

A.11.2.1.8.1	Test Purpose and Environment	2437

A.11.2.1.8.2	Test Requirements	2442

A.11.2.2	RRC connection mobility control	2442

A.11.2.2.1	RRC re-establishment	2442

A.11.2.2.1.1	Intra-frequency RRC Re-establishment with CCA in FR1	2442

A.11.2.2.1.2	Inter-frequency RRC Re-establishment with CCA in FR1	2445

A.11.2.2.1.4	Inter-frequency RRC Re-establishment from NR FR1 carrier without CCA to NR FR1 carrier under CCA	2453

A.11.2.2.2	Random Access	2456

A.11.2.2.2.1	4-step RA type contention-based random access for NR PCell with CCA	2456

A.11.2.2.2.1.1	Test Purpose and Environment	2456

A.11.2.2.2.1.2	Test Requirements	2460

A.11.2.2.2.1.2.1	Random Access Preamble Transmission	2460

A.11.2.2.2.1.2.2	Random Access Response Reception	2460

A.11.2.2.2.1.2.3	No Random Access Response Reception	2461

A.11.2.2.2.1.2.4	Receiving an UL grant for msg3 retransmission	2461

A.11.2.2.2.1.2.5	Reception of an Incorrect Message over Temporary C-RNTI	2461

A.11.2.2.2.1.2.6	Reception of a Correct Message over Temporary C-RNTI	2462

A.11.2.2.2.1.2.7	Contention Resolution Timer expiry	2462

A.11.2.2.2.2	4-step RA type non-contention based random access for NR PSCell with CCA	2462

A.11.2.2.2.2.1	Test Purpose and Environment	2462

A.11.2.2.2.2.2	Test Requirements	2465

A.11.2.2.2.2.2.1	SSB-based Random Access Preamble Transmission	2465

A.11.2.2.2.2.2.2	Random Access Response Reception	2466

A.11.2.2.2.2.2.3	No Random Access Response Reception	2466

A.11.2.2.2.3	2-step RA type contention-based random access for NR PCell with CCA	2466

A.11.2.2.2.3.1	Test Purpose and Environment	2466

A.11.2.2.2.3.2	Test Requirements	2470

A.11.2.2.2.3.2.1	MsgA Transmission	2470

A.11.2.2.2.3.2.2	MsgB Reception	2470

A.11.2.2.2.3.2.3	No MsgB Reception	2471

A.11.2.2.2.4	2-step RA type non-contention-based random access for NR PCell with CCA	2471

A.11.2.2.2.4.1	Test Purpose and Environment	2471

A.11.2.2.2.4.2	Test Requirements	2475

A.11.2.2.2.4.2.1	MsgA Transmission	2475

A.11.2.2.2.4.2.2	MsgB Reception	2476

A.11.2.2.2.4.2.3	No MsgB Reception	2476

A.11.2.2.3	RRC connection release with redirection	2477

A.11.2.2.3.1	Redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA	2477

A.11.2.2.3.2	Redirection from NR FR1 carrier without CCA to NR FR1 carrier with CCA	2481

A.11.3	Timing	2485

A.11.3.1	UE transmit timing	2485

A.11.3.1.1	UE Transmit Timing Test with PCell under DL CCA	2485

A.11.3.1.1.1	Test Purpose and environment	2485

A.11.3.1.1.2	Test requirements	2489

A.11.3.2	UE timing advance	2490

A.11.3.2.1	UE Timing Advance Adjustment Accuracy with PCell under DL CCA	2490

A.11.3.2.1.1	Test Purpose and Environment	2490

A.11.3.2.1.2	Test Parameters	2490

A.11.3.2.1.3	Test Requirements	2493

A.11.4	Signalling characteristics	2493

A.11.4.1	Radio link monitoring	2493

A.11.4.1.1	Introduction	2493

A.11.4.1.2	Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	2494

A.11.4.1.2.1	Test purpose and environment	2494

A.11.4.1.2.2	Test requirements	2498

A.11.4.1.3	Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	2498

A.11.4.1.3.1	Test purpose and environment	2498

A.11.4.1.3.2	Test requirements	2504

A.11.4.1.4	Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode	2504

A.11.4.1.4.1	Test purpose and environment	2504

A.11.4.1.4.2	Test requirements	2504

A.11.4.1.5	Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode	2504

A.11.4.1.5.1	Test purpose and environment	2504

A.11.4.1.5.2	Test requirements	2504

A.11.4.2	Interruption	2505

A.11.4.2.1	NR interruptions during Scell operations with CCA on PCell and SCell	2505

A.11.4.2.1.1 Test Purpose and Environment	2505

A.11.4.2.1.2	Test Requirements	2509

A.11.4.3	SCell activation and deactivation delay	2509

A.11.4.3.1	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 160 ms SCell measurement cycle	2509

A.11.4.3.1.1	Test Purpose and Environment	2509

A.11.4.3.1.2	Test Requirements	2514

A.11.4.3.2	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 320 ms SCell measurement cycle	2514

A.11.4.3.2.1	Test Purpose and Environment	2514

A.11.4.3.2.2	Test Requirements	2515

A.11.4.3.3	SCell Activation and Deactivation of unknown SCell with PCell and SCell under CCA	2515

A.11.4.3.3.1	Test Purpose and Environment	2515

A.11.4.3.3.2	Test Requirements	2516

A.11.4.4	Beam failure detection and link recovery procedures	2516

A.11.4.4.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	2516

A.11.4.4.1.1	Test Purpose and Environment	2516

A.11.4.4.1.2	Test Requirements	2522

A.11.4.4.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	2522

A.11.4.4.2.1	Test Purpose and Environment	2522

A.11.4.4.2.2	Test Requirements	2529

A.11.4.5	Active BWP switching	2529

A.11.4.5.1	UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA	2529

A.11.4.5.1.1	Test Purpose and Environment	2529

A.11.4.5.1.2	Test Requirements	2534

A.11.4.5.2	DCI-based and Timer-based Active BWP Switch	2535

A.11.4.5.2.1	NR FR1- NR FR1 DL active BWP switch of PCell with non-DRX in SA	2535

A.11.4.5.2.2	NR FR1 DL active BWP switch with non-DRX in SA	2538

A.11.4.5.3	RRC-based Active BWP Switch	2541

A.11.4.5.3.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	2541

A.11.4.6	Active TCI state switching delay	2544

A.11.5	Measurement procedure	2544

A.11.5.1	Intra-frequency measurements	2544

A.11.5.1.1	Event-triggered reporting tests on PCC without gaps under non-DRX	2544

A.11.5.1.1.1	Test purpose and environment	2544

A.11.5.1.1.2	Test parameters	2544

A.11.5.1.1.3	Test Requirements	2545

A.11.5.1.2	Event-triggered reporting tests on PCC without gaps under DRX	2545

A.11.5.1.2.1	Test purpose and environment	2545

A.11.5.1.2.2	Test parameters	2545

A.11.5.1.2.3	Test Requirements	2545

A.11.5.1.3	Event-triggered reporting tests on PCC with per-UE gaps under non-DRX	2546

A.11.5.1.3.1	Test purpose and environment	2546

A.11.5.1.3.2	Test parameters	2546

A.11.5.1.3.3	Test Requirements	2546

A.11.5.1.4	Event-triggered reporting tests on PCC with per-UE gaps under DRX	2546

A.11.5.1.4.1	Test purpose and environment	2546

A.11.5.1.4.2	Test parameters	2546

A.11.5.1.4.3	Test Requirements	2546

A.11.5.1.5	Event-triggered reporting tests on SCC without gaps under non-DRX	2547

A.11.5.1.5.1	Test purpose and environment	2547

A.11.5.1.5.2	Test parameters	2547

A.11.5.1.6	Event-triggered reporting tests on SCC without gaps under DRX	2547

A.11.5.1.6.1	Test purpose and environment	2547

A.11.5.1.6.2	Test parameters	2547

A.11.5.1.6.3	Test Requirements	2547

A.11.5.1.7	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	2548

A.11.5.1.7.1	Test purpose and environment	2548

A.11.5.1.7.2	Test parameters	2548

A.11.5.1.7.3	Test Requirements	2548

A.11.5.1.8	Event-triggered reporting tests on SCC with per-UE gaps under DRX	2548

A.11.5.1.8.1	Test purpose and environment	2548

A.11.5.1.8.2	Test parameters	2548

A.11.5.1.8.3	Test Requirements	2548

A.11.5.1.9	RSSI measurement reporting on PCC	2549

A.11.5.1.9.1	Test purpose and environment	2549

A.11.5.1.9.2	Test parameters	2549

A.11.5.1.10	Channel occupancy measurement reporting on PCC	2549

A.11.5.1.10.1	Test purpose and environment	2549

A.11.5.1.10.2	Test parameters	2549

A.11.5.1.11	RSSI measurement reporting on SCC	2549

A.11.5.1.11.1	Test purpose and environment	2549

A.11.5.1.11.2	Test parameters	2550

A.11.5.1.12	Channel occupancy measurement reporting on SCC	2550

A.11.5.1.12.1	Test purpose and environment	2550

A.11.5.1.12.2	Test parameters	2550

A.11.5.2	Inter-frequency measurements	2550

A.11.5.2.1	RSSI measurement reporting	2550

A.11.5.2.1.1	Test purpose and environment	2550

A.11.5.2.1.2	Test parameters	2551

A.11.5.2.2	Channel occupancy measurement reporting	2551

A.11.5.2.2.1	Test purpose and environment	2551

A.11.5.2.2.2	Test parameters	2551

A.11.5.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	2551

A.11.5.2.3.1	Test Purpose and Environment	2551

A.11.5.2.3.2	Test Requirements	2555

A.11.5.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	2556

A.11.5.2.4.1	Test Purpose and Environment	2556

A.11.5.2.4.2	Test Requirements	2560

A.11.5.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	2561

A.11.5.2.5.1	Test Purpose and Environment	2561

A.11.5.2.5.2	Test Requirements	2565

A.11.5.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	2566

A.11.5.2.6.1	Test Purpose and Environment	2566

A.11.5.2.6.2	Test Requirements	2570

A.11.5.2.7	Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2571

A.11.5.2.7.1	Test Purpose and Environment	2571

A.11.5.2.7.2	Test Requirements	2575

A.11.5.2.8	Event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2576

A.11.5.2.8.1	Test Purpose and Environment	2576

A.11.5.2.8.2	Test Requirements	2581

A.11.5.2.9	Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2581

A.11.5.2.9.1	Test Purpose and Environment	2581

A.11.5.2.9.2	Test Requirements	2586

A.11.5.2.10	Event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2587

A.11.5.2.10.1	Test Purpose and Environment	2587

A.11.5.2.10.2	Test Requirements	2592

A.11.5.3	Inter-RAT E-UTRAN measurements	2593

A.11.5.4	L1-RSRP measurements for beam reporting	2593

A.11.5.4.1	SSB based L1-RSRP measurement when DRX is not used	2593

A.11.5.4.1.1	Test Purpose and Environment	2593

A.11.5.4.1.2	Test parameters	2593

A.11.5.4.1.3	Test Requirements	2595

A.11.5.4.2	SSB based L1-RSRP measurement when DRX is used	2595

A.11.5.4.2.1	Test Purpose and Environment	2595

A.11.5.4.2.2	Test parameters	2596

A.11.5.4.2.3	Test Requirements	2598

A.11.5.4.3	SSB based L1-RSRP measurement on SCC when DRX is not used	2598

A.11.5.4.3.1	Test Purpose and Environment	2598

A.11.5.4.3.2	Test parameters	2599

A.11.5.4.3.3	Test Requirements	2602

A.11.5.4.4	SSB based L1-RSRP measurement on SCC when DRX is used	2603

A.11.5.4.4.1	Test Purpose and Environment	2603

A.11.5.4.4.2	Test parameters	2603

A.11.5.4.4.3	Test Requirements	2606

A.11.6	Measurement performance	2607

A.11.6.1	SS-RSRP	2607

A.11.6.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	2607

A.11.6.1.1.1	Test Purpose and Environment	2607

A.11.6.1.1.2	Test parameters	2607

A.11.6.1.1.3	Test Requirements	2609

A.11.6.1.2	Intra-frequency measurement accuracy on SCC on a carrier frequency with CCA	2609

A.11.6.1.2.1	Test Purpose and Environment	2609

A.11.6.1.2.2	Test parameters	2609

A.11.6.1.2.3	Test Requirements	2611

A.11.6.2	SS-RSRQ	2611

A.11.6.3	SS-SINR	2611

A.11.6.4	L1-RSRP measurement for beam reporting with CCA serving cell	2611

A.11.6.4.1	SSB based L1-RSRP measurement	2611

A.11.6.4.1.1	Test Purpose and Environment	2611

A.11.6.4.1.2	Test parameters	2611

A.11.6.4.1.3	Test Requirements	2614

A.11.6.5	RSSI	2614

A.11.6.5.1 	Intra-frequency RSSI measurement accuracy on PCC with CCA	2614

A.11.6.5.1.1	Test Purpose and Environment	2614

A.11.6.5.1.2	Test parameters	2614

A.11.6.5.1.3	Test Requirements	2617

A.11.6.5.2 	Intra-frequency RSSI measurement accuracy on SCC with CCA	2617

A.11.6.5.2.1	Test Purpose and Environment	2617

A.11.6.5.2.2	Test parameters	2617

A.11.6.5.2.3	Test Requirements	2620

A.11.6.5.3	Inter-frequency RSSI measurement accuracy on a carrier with CCA	2620

A.11.6.5.3.1	Test Purpose and Environment	2620

A.11.6.5.3.2	Test parameters	2620

A.11.6.5.3.3	Test Requirements	2623

A.11.6.6	Channel occupancy	2623

A.11.6.6.1	Intra-frequency channel occupancy measurement accuracy on PCC with CCA	2623

A.11.6.6.1.1	Test Purpose and Environment	2623

A.11.6.6.1.2	Test parameters	2623

A.11.6.6.1.3	Test Requirements	2626

A.11.6.6.2	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	2626

A.11.6.6.2.1	Test Purpose and Environment	2626

A.11.6.6.2.2	Test parameters	2626

A.11.6.6.2.3	Test Requirements	2629

A.11.6.6.3	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	2629

A.11.6.6.3.1	Test Purpose and Environment	2629

A.11.6.6.3.2	Test parameters	2629

A.11.6.6.3.3	Test Requirements	2632

A.11.6.7	E-UTRAN RSRP	2632

A.11.6.8	E-UTRAN RSRQ	2632

A.11.6.9	E-UTRAN SINR	2632

A.12	E-UTRA Standalone Tests with at Least One NR Cell under CCA	2632

A.12.1	RRC_IDLE state mobility	2632

A.12.1.1	Inter-RAT cell re-selection to NR on a carrier frequency with CCA	2632

A.12.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1 when target cell is subject to CCA	2632

A.12.1.1.1.1	Test Purpose and Environment	2632

A.12.1.1.1.2	Test Requirements	2637

A.12.2	RRC_CONNECTED state mobility	2638

A.12.2.1	Handover	2638

A.12.2.1.1	E-UTRAN - NR with CCA handover	2638

A.12.2.1.1.1	Test Purpose and Environment	2638

A.12.2.1.1.2	Test Requirements	2643

A.12.3	Signalling characteristics	2643

A.12.3.1	Interruptions	2643

A.12.4	Measurement procedure	2643

A.12.4.1	E-UTRANNR inter-RAT SFTD measurements	2643

A.12.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay with NR under CCA in non-DRX	2643

A.12.4.1.1.1	Test Purpose and Environment	2643

A.12.4.1.1.2	Test Requirements	2647

A.12.4.2	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	2647

A.12.4.2.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2647

A.12.4.2.1.1	Test Purpose and Environment	2647

A.12.4.2.1.2	Test Requirements	2651

A.12.4.2.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2652

A.12.4.2.2.1	Test Purpose and Environment	2652

A.12.4.2.2.2	Test Requirements	2656

A.12.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2656

A.12.4.2.3.1	Test Purpose and Environment	2656

A.12.4.2.3.2	Test Requirements	2660

A.12.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2661

A.12.4.2.4.1	Test Purpose and Environment	2661

A.12.4.2.4.2	Test Requirements	2665

A.12.4.2.5	RSSI measurement reporting	2665

A.12.4.2.5.1	Test purpose and environment	2665

A.12.4.2.5.2	Test parameters	2665

A.12.4.2.6	Channel occupancy measurement reporting	2666

A.12.4.2.6.1	Test purpose and environment	2666

A.12.4.2.6.2	Test parameters	2666

A.12.5	Measurement performance	2666

A.12.5.1	E-UTRANNR SFTD	2666

A.12.5.1.1	Inter-RAT SFTD accuracy with NR target cell under CCA	2666

A.12.5.1.1.1	Test Purpose	2666

A.12.5.1.1.2	Test Environment	2667

A.12.5.1.1.3	Test Requirements	2672

A.12.5.2	E-UTRANNR SS-RSRP	2672

A.12.5.3	E-UTRANNR SS-RSRQ	2672

A.12.5.4	E-UTRANNR SS-SINR	2672

A.12.5.5	E-UTRANNR RSSI	2672

A.12.5.6	E-UTRANNR channel occupancy	2672

A.13	NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	2672

A.13.1	Timing	2672

A.13.1.1	UE transmit timing	2672

A.13.1.2	Timing advance	2672

A.13.2	Signalling characteristics	2672

A.13.2.1	Interruption	2672

A.13.2.1.1	NR interruptions during SCell operations with CCA on SCell	2672

A.13.2.1.1.1	Test Purpose and Environment	2672

A.13.2.1.1.2	Test Requirements	2676

A.13.2.2	SCell activation and deactivation delay	2676

A.13.2.2.1	SCell Activation and Deactivation of known SCell under CCA, 160 ms SCell measurement cycle	2676

A.13.2.2.1.1	Test Purpose and Environment	2676

A.13.2.2.1.2	Test Requirements	2681

A.13.2.2.2 SCell Activation and Deactivation of known SCell under CCA, 320 ms SCell measurement cycle	2681

A.13.2.2.2.1	Test Purpose and Environment	2681

A.13.2.2.2.2	Test Requirements	2682

A.13.2.2.3	SCell Activation and Deactivation of unknown SCell under CCA	2682

A.13.2.2.3.1	Test Purpose and Environment	2682

A.13.2.2.3.2	Test Requirements	2683

A.13.2.3	Active TCI state switching delay	2683

A.13.3	Measurement procedure	2683

A.13.3.1	Intra-frequency measurements	2683

A.13.3.1.1	Event-triggered reporting tests on SCC without gaps under non-DRX	2683

A.13.3.1.1.1	Test purpose and environment	2683

A.13.3.1.1.2	Test parameters	2683

A.13.3.1.1.3	Test Requirements	2688

A.13.3.1.2	Event-triggered reporting tests on SCC without gaps under DRX	2688

A.13.3.1.2.1	Test purpose and environment	2688

A.13.3.1.2.2	Test parameters	2688

A.13.3.1.2.3	Test Requirements	2692

A.13.3.1.3	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	2692

A.13.3.1.3.1	Test purpose and environment	2692

A.13.3.1.3.2	Test parameters	2692

A.13.3.1.3.3	Test Requirements	2697

A.13.3.1.4	Event-triggered reporting tests on SCC with per-UE gaps under DRX	2697

A.13.3.1.4.1	Test purpose and environment	2697

A.13.3.1.4.2	Test parameters	2697

A.13.3.1.4.3	Test Requirements	2702

A.13.3.1.5	RSSI measurement reporting on SCC	2702

A.13.3.1.5.1	Test purpose and environment	2702

A.13.3.1.5.2	Test parameters	2702

A.13.3.1.6	Channel occupancy measurement reporting on SCC	2703

A.13.3.1.6.1	Test purpose and environment	2703

A.13.3.1.6.2	Test parameters	2704

A.13.3.2	Inter-frequency measurements	2704

A.13.3.2.1	RSSI measurement reporting	2704

A.13.3.2.1.1	Test purpose and environment	2704

A.13.3.2.1.2	Test parameters	2704

A.13.3.2.2	Channel occupancy measurement reporting	2705

A.13.3.2.2.1	Test purpose and environment	2705

A.13.3.2.2.2	Test parameters	2705

A.13.3.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	2705

A.13.3.2.3.1	Test Purpose and Environment	2705

A.13.3.2.3.2	Test Requirements	2710

A.13.3.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	2710

A.13.3.2.4.1	Test Purpose and Environment	2710

A.13.3.2.4.2	Test Requirements	2715

A.13.3.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	2716

A.13.3.2.5.1	Test Purpose and Environment	2716

A.13.3.2.5.2	Test Requirements	2721

A.13.3.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	2721

A.13.3.2.6.1	Test Purpose and Environment	2721

A.13.3.2.6.2	Test Requirements	2727

A.13.3.3	L1-RSRP measurements for beam reporting	2728

A.13.3.3.1	SSB based L1-RSRP measurement when DRX is not used	2728

A.13.3.3.1.1	Test Purpose and Environment	2728

A.13.3.3.1.2	Test parameters	2728

A.13.3.3.1.3	Test Requirements	2732

A.13.3.3.2	SSB based L1-RSRP measurement when DRX is used	2732

A.13.3.3.2.1	Test Purpose and Environment	2732

A.13.3.3.2.2	Test parameters	2732

A.13.3.3.2.3	Test Requirements	2736

A.13.4	Measurement performance	2736

A.13.4.1	SS-RSRP	2736

A.13.4.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	2736

A.13.4.1.1.1	Test Purpose and Environment	2736

A.13.4.1.1.2	Test parameters	2736

A.13.4.1.1.3	Test Requirements	2738

A.13.4.2	SS-RSRQ	2738

A.13.4.3	SS-SINR	2738

A.13.4.4	L1-RSRP measurement for beam reporting with CCA serving cell	2738

A.13.4.4.1	SSB based L1-RSRP measurement	2738

A.13.4.4.1.1	Test Purpose and Environment	2738

A.13.4.4.1.2	Test parameters	2738

A.13.4.4.1.3	Test Requirements	2743

A.13.4.5	RSSI	2743

A.13.4.5.1 	Intra-frequency RSSI measurement accuracy on a carrier with CCA	2743

A.13.4.5.1.1	Test Purpose and Environment	2743

A.13.4.5.1.2	Test parameters	2743

A.13.4.5.1.3	Test Requirements	2746

A.13.4.5.2	Inter-frequency RSSI measurement accuracy on a carrier with CCA	2746

A.13.4.5.2.1	Test Purpose and Environment	2746

A.13.4.5.2.2	Test parameters	2746

A.13.4.5.2.3	Test Requirements	2750

A.13.4.6	Channel occupancy	2750

A.13.4.6.1	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	2750

A.13.4.6.1.1	Test Purpose and Environment	2750

A.13.4.6.1.2	Test parameters	2750

A.13.4.6.1.3	Test Requirements	2754

A.13.4.6.2	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	2754

A.13.4.6.2.1	Test Purpose and Environment	2754

A.13.4.6.2.2	Test parameters	2754

A.13.4.6.2.3	Test Requirements	2758

Annex B (normative): Conditions for RRM requirements applicability for operating bands	1787

B.1	Conditions for NR RRC_IDLE state mobility	1787

B.1.1	Introduction	1787

B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection	1787

B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection	1788

B.2	Conditions for UE measurements procedures and performance requirements in RRC_CONNECTED state	1788

B.2.1	Introduction	1788

B.2.1.1	General	1788

B.2.1.2	Derivation of Minimum SSB_RP values for FR1	1789

B.2.1.3	Derivation of Minimum SSB_RP values for FR2	1789

B.2.1.3.1	Minimum SSB_RP values for Rx Beam Peak angle of arrival	1789

B.2.1.3.2	Minimum SSB_RP values for angle of arrival within Spherical coverage	1790

B.2.1.4	Gain to SS-RSRP and CSI-RSRP measurement point for FR1	1790

B.2.1.5	Gain to SS-RSRP and CSI-RSRP measurement point for FR2	1790

B.2.1.5.1	Gain to SS-RSRP and CSI-RSRP measurement point for Rx Beam Peak angle of arrival	1790

B.2.1.6	Gain to PRS-RSRP measurement point for FR2	1791

B.2.1.6.1	Gain to PRS-RSRP measurement point for Rx Beam Peak angle of arrival	1791

B.2.2	Conditions for NR intra-frequency measurements	1792

B.2.3	Conditions for NR inter-frequency measurements	1793

B.2.4	Conditions for NR L1-RSRP reporting	1794

B.2.4.1	Conditions for SSB based L1-RSRP reporting	1794

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting	1795

B.2.5	Conditions for RRC connection release with redirection to NR	1796

B.2.6	Void	1797

B.2.6.1	Void	1797

B.2.6.2	Void	1797

B.2.8	Conditions for NR L1-SINR reporting	1798

B.2.8.1	Conditions for L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	1798

B.2.8.2	Conditions for L1-SINR reporting with SSB based CMR and dedicated IMR configured	1799

B.2.8.2.1	L1-SINR reporting with SSB based CMR and dedicated ZP-IMR configured	1799

B.2.8.2.2	L1-SINR reporting with SSB based CMR and dedicated NZP-IMR configured	1800

B.2.8.3	Conditions for L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	1801

B.2.8.3.1	L1-SINR reporting with CSI-RS based CMR and dedicated ZP-IMR configured	1801

B.2.8.3.2	L1-SINR reporting with CSI-RS based CMR and dedicated NZP-IMR configured	1802

B.2.9	Conditions for NR intra-frequency measurements under CCA	1804

B.2.10	Conditions for NR inter-frequency measurements under CCA	1804

B.2.11	Conditions for NR L1-RSRP reporting under CCA	1804

B.2.11.1	Conditions for SSB based L1-RSRP reporting	1804

B.2.12	Conditions for NR CSI-RS based intra-frequency measurements	1805

B.2.13	Conditions for NR CSI-RS based inter-frequency measurements	1806

B.2.14	Conditions for NR PRS-based measurements	1807

B.3	RRM Requirements Exceptions	1809

B.3.1	Introduction	1809

B.3.2	Receiver sensitivity relaxation for CA	1809

B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1	1809

B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1	1809

B.3.2.2.1	Inter-band carrier aggregation	1809

B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA	1809

B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	1809

B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2	1810

B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2	1810

B.3.2.4.1	Intra-band contiguous carrier aggregation	1810

B.3.2.4.2	Intra-band non-contiguous carrier aggregation	1810

B.3.3	Receiver sensitivity relaxation for DC	1810

B.3.3.1	Receiver sensitivity relaxation for EN-DC	1810

B.3.3.2	Receiver sensitivity relaxation for NE-DC	1810

B.3.4	Receiver sensitivity relaxation for SUL	1810

B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1	1810

B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1	1810

B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL	1810

B.4	Conditions for V2X	1811

B.4.1	Test parameters for GNSS signals	1811

B.4.2	Conditions for PSBCH-RSRP Accuracy Requirements	1811

B.4.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	1811

B.4.4	Conditions for L1 SL-RSRP Accuracy Requirements	1811

Annex C (informative): Change history	1813