| 3GPP TS 38.391 V0.0.2 (2025-05) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Ambient IoT Medium Access Control Protocol specification (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions, symbols and abbreviations	6

3.1	Definitions	6

3.2	Abbreviations	6

4	General	7

4.1	Introduction	7

4.2	A-IoT MAC architecture	7

4.3	Services	7

4.3.1	Services provided to upper layers	7

4.3.2	Services expected from physical layer	7

4.4	Functions	8

5	A-IoT MAC procedures	8

5.1	General	8

5.2	A-IoT paging	8

5.3	A-IoT random access procedure	9

5.3.1	Contention-Based Random Access procedure	9

5.3.1.1	Selection of access occasion for D2R transmission of Random ID message	9

5.3.1.2	Transmission of Random ID message	9

5.3.1.3	Reception of Random ID Response message	9

5.3.2	Contention-Free Random Access procedure	10

5.4	A-IoT upper layer data transmission	10

5.4.1	D2R message transmission	10

5.4.2	R2D message reception	11

5.4.3	Segmentation	11

5.5	Failure detection	12

6	Protocol Data Units, formats and parameters	12

6.1	Protocol Data Units	12

6.1.1	General	12

6.2	A-IoT MAC messages	13

6.2.1	R2D messages	13

6.2.1.1	A-IoT Paging message	13

6.2.1.2	Access Occasion Trigger message	13

6.2.1.3	Random ID Response message (Msg2 in CBRA)	13

6.2.1.4	R2D Upper Layer Data Transfer message	14

6.2.2	D2R messages	14

6.2.2.1	Random ID message (Msg1 in CBRA)	14

6.2.2.2	D2R Upper Layer Data Transfer message	14

Annex <X> (informative): Change history	15

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the Medium Access Control (MAC) protocol of Ambient IoT.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.291: "Ambient IoT Physical layer".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Device:

Reader:

Access occasion: A time-frequency resource for device(s) to transmit Msg1 (i.e., the Random ID message) during a CBRA procedure.

AS ID: The AS layer identifier to address the specific device for R2D reception and D2R scheduling.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

A-IoT	Ambient IoT

CBRA	Contention-Based Random Access

CFRA	Contention-Free Random Access

D2R	Device to reader

IoT	Internet of Things

PDRCH	Physical D2R channel

PRDCH	Physical R2D channel

R2D	Reader to device

# 4	General

## 4.1	Introduction

This clause describes the A-IoT MAC architecture and the A-IoT MAC entity of the device from a functional point of view.

## 4.2	A-IoT MAC architecture

Figure 4.2-1 illustrates a model of the A-IoT MAC entity; and it does not restrict implementations.

The A-IoT MAC entity of the device handles the data received or to be transmitted via the physical channels, i.e., PRDCH and PDRCH, as specified in TS 38.291 [2].

Figure 4.2-1: A-IoT MAC structure overview

Editor’s Note:	FFS whether the concept of transport channel is needed for A-IoT. FFS whether logical channel or SAP is used between A-IoT MAC and upper layers.

## 4.3	Services

### 4.3.1	Services provided to upper layers

The A-IoT MAC layer provides the following services to upper layers:

-	data transfer;

### 4.3.2	Services expected from physical layer

The A-IoT MAC layer expects the following services from the physical layer:

-	data transfer;

## 4.4	Functions

The A-IoT MAC layer supports the following A-IoT MAC functions:

-	constructing MAC PDUs to be mapped onto D2R transport blocks and delivered to the physical layer;

-	receiving MAC PDUs from R2D transport blocks delivered from the physical layer;

-	message type determination;

-	paging;

-	radio resource selection;

-	random access;

-	transfer of upper layer data;

-	D2R segmentation;

-	failure detection.

# 5	A-IoT MAC procedures

## 5.1	General

The clause describes the A-IoT MAC procedures.

When the device is powered on, the device starts monitoring the R2D messages on PRDCH, as specified in TS 38.291 [2], in order to perform the corresponding A-IoT MAC procedures.

## 5.2	A-IoT paging

The purpose of this procedure is to transmit A-IoT Paging message to one or more devices. The reader may include the Paging ID field to select a specific device or a group of devices, or include no Paging ID field to select all devices.

The device monitors the A-IoT Paging message, and determines whether the device is selected and initiates the random access procedure.

Upon receiving the A-IoT Paging message, the A-IoT MAC entity shall:

1>	if the Transaction ID field included in the A-IoT Paging message is the same as the value stored by the device, if any:

2>	if the previous procedure associated with the received value indicated in the Transaction ID field is determined as failed as specified in clause 5.5:

3>	consider the device is selected by this A-IoT Paging message;

1>	else:

2>	store the received value in Transaction ID field or replace the previously stored Transaction ID with the current received value;

Editor’s Note:	FFS device behaviour if multiple requests are received in parallel (if needed). FFS which solution if any for device behavior if it gets a new service request while one procedure is still ongoing or leave it to implementation.

Editor’s Note:	For CFRA, NACK feedback and re-access is not supported. FFS how to achieve. FFS on end of procedure.

2>	if the A-IoT Paging message does not include Paging ID field:

3>	consider the device is selected by this A-IoT Paging message and indicate to the upper layers;

2>	else:

3>	forward the value of the Paging ID field to the upper layers;

3>	if the upper layers indicate that the Paging ID is matched:

4>	consider the device is selected by this A-IoT Paging message;

1>	if the device is selected by this A-IoT Paging message:

2>	release the stored AS ID if any;

2>	if the RA Type field in the A-IoT Paging message indicates CBRA:

3>	process the received D2R Scheduling Info field in A-IoT Paging message as specified in clause 5.3.1.1;

3>	perform Contention-Based Random Access procedure as specified in clause 5.3.1;

2>	else (the RA Type field in the A-IoT Paging message indicates CFRA):

3>	apply the received D2R Scheduling Info field in A-IoT Paging message and indicate it to the physical layer;

3>	perform Contention-Free Random Access procedure as specified in clause 5.3.2;

Editor’s Note:	FFS other cases for release ASID to avoid keeping it indefinitely.

## 5.3	A-IoT random access procedure

### 5.3.1	Contention-Based Random Access procedure

#### 5.3.1.1	Selection of access occasion for D2R transmission of Random ID message

If Contention-Based Random Access procedure is initiated according to clause 5.2, the A-IoT MAC entity shall:

1>	randomly select an access occasion for transmission of the Random ID message among the access occasions configured in A-IoT Paging message;

1>	apply the D2R Scheduling Info received from the A-IoT Paging message for the selected access occasion;

1>	initiate the transmission of Random ID message, as specified in clause 5.3.1.2.

Editor’s Note: More details may be added later according to further agreement if any, e.g., how the device determine the selected access occasion based on the Access Occasion Trigger message.

#### 5.3.1.2	Transmission of Random ID message

The A-IoT MAC entity shall:

1>	generate a 16-bit random number 'j' in the range: 0 ≤ j < 216;

1>	set the Random ID field to the ‘j’ in the Random ID message;

1>	instruct the physical layer to transmit the Random ID message using the selected access occasion.

#### 5.3.1.3	Reception of Random ID Response message

Once the Random ID message is transmitted, and if a Random ID Response message is received, the A-IoT MAC entity shall:

1>	if the value indicated in Echoed Random ID field in Random ID Response message is identical to the value of the Random ID field in the transmitted Random ID message:

2>	consider this Random Access procedure is successfully completed;

2>	if the Assigned AS ID field corresponding to the Echoed Random ID field is included:

3>	set AS ID to the value indicated in the Assigned AS ID field and store the AS ID;

2>	else:

3>	set AS ID to the value of the Random ID field in the transmitted Random ID message and store the AS ID;

2>	apply the received D2R Scheduling Info field in Random ID Response message and indicate it to the physical layer;

2>	initiate the D2R message transmission as specified in clause 5.4.1.

### 5.3.2	Contention-Free Random Access procedure

If Contention-Free Random Access procedure is initiated according to clause 5.2, the A-IoT MAC entity shall:

1>	initiate the D2R message transmission as specified in clause 5.4.1.

## 5.4	A-IoT upper layer data transmission

### 5.4.1	D2R message transmission

Once the D2R Scheduling Info for D2R upper layer data transmission is received in the A-IoT Paging message or the Random ID Response message or the R2D Upper Layer Data Transfer message, the A-IoT MAC entity shall:

1>	if upper layer data is available to be transmitted:

2>	if the size of the resulting MAC PDU including the total UL data is smaller than or equal to the resource size given by the D2R Scheduling Info:

3>	generate the D2R Upper Layer Data Transfer message, including:

4>	include the Data SDU field;

4>	set the More Data Indication field to value 0;

4>	if the size of the resulting MAC PDU including the total UL data is smaller than the resource size given by the D2R Scheduling Info:

5>	include the MAC Padding field;

3>	instruct the physical layer to transmit the generated D2R Upper Layer Data Transfer message;

2>	else (the size of the resulting MAC PDU including the total UL data is larger than the resource size given by the D2R Scheduling Info):

3>	the upper layer data SDU is to be segmented according to clause 5.4.3;

Editor’s Note: FFS whether write command type may cause a case of ‘no upper layer data is available for a D2R scheduling’ due to long writing time.

NOTE:	It is up to reader’s implementation to avoid segmentation for the D2R Upper Layer Data Transfer message for device ID reporting.

### 5.4.2	R2D message reception

Once a R2D message is received, the A-IoT MAC entity shall:

1>	if the device has stored an AS ID and the R2D message is addressed to the stored AS ID:

2>	forward the upper layer data SDU to upper layers;

2>	apply the D2R Scheduling Info field and indicate it to the physical layer for the following D2R message transmission, as specified in 5.4.1;

1>	else:

2>	if the device has no stored AS ID; and

2>	if the R2D message is the R2D Upper Layer Data Transfer message; and

2>	if CFRA procedure has been performed in the current procedure:

3>	set AS ID to the value indicated in the AS ID field and store the AS ID;

3>	forward the upper layer data SDU to upper layers;

3>	apply the D2R Scheduling Info field and indicate it to the physical layer for the following D2R message transmission, as specified in 5.4.1.

Editor’s Note:	FFS whether the reader always includes the command for retransmission of segments.

### 5.4.3	Segmentation

When an upper layer data SDU is to be segmented according to D2R upper layer data transmission procedure in clause 5.4.1, the A-IoT MAC entity performs this segmentation procedure for the original upper layer data SDU.

The A-IoT MAC entity shall:

1>	if the D2R Scheduling Info is received in the R2D Upper Layer Data Transfer message:

2>	generate the D2R Upper Layer Data Transfer message for this segment according to the D2R Scheduling Info as follows:

3>	set the Data SDU field to include the segment which starts from the (x+1)th byte, indicated by the Received Data Size field, i.e., x bytes, of the original upper layer data SDU;

Editor’s Note:	FFS whether offset zero is always included for the retransmission of the first segment/unsegmented D2R message.

3>	if the segment is the last segment of the original upper layer data SDU:

4>	set More Data Indication field to value 0;

4>	if the size of the resulting MAC PDU including the segment is expected to be smaller than the resource size given by the D2R Scheduling Info:

5>	include the MAC Padding field;

3>	else:

4>	set More Data Indication field to value 1;

2>	instruct the physical layer to transmit the D2R Upper Layer Data Transfer message.

Editor’s Note:	To be updated after concluding the format of the Command message (e.g. EN in 5.4.2).

## 5.5	Failure detection

The A-IoT MAC entity shall:

1>	if CBRA procedure fails; or

1>	if the device receives NACK feedback, before subsequent R2D message:

2>	consider that the current procedure associated with the stored Transaction ID failed.

Editor’s Note:	FFS how to determine failure of CBRA procedure.

Editor’s Note:	FFS whether subsequent R2D message is trigger message or paging. FFS details on NACK feedback, including whether we need an explicit message.

Editor’s Note:	FFS how to capture the agreement “For CBRA, as a baseline, NACK based mechanism is applied only to the Msg3.”.

# 6	Protocol Data Units, formats and parameters

## 6.1	Protocol Data Units

### 6.1.1	General

An A-IoT MAC Protocol Data Unit (PDU) is the data unit format in which the A-IoT MAC message is encapsulated for transmission through the lower layer of the A-IoT protocol stack. An A-IoT MAC PDU is a bit string. The contents of each A-IoT MAC message are specified in clause 6.2 using tables to specify the fields in the message. In the tables, the most significant bit is the leftmost bit of the first line of the table, the least significant bit is the rightmost bit on the last line of the table, and more generally the bit string is to be read from left to right and then in the reading order of the lines. The bit order of each parameter field within a MAC PDU is represented with the first and most significant bit in the leftmost bit and the last and least significant bit in the rightmost bit.

An A-IoT MAC SDU is a bit string that is byte aligned (i.e., multiple of 8 bits) in length. A MAC SDU is included into a MAC PDU from the first bit onward.

The R2D message type represents the set of A-IoT MAC messages that are sent from the reader to the device on the PRDCH. The values of R2D message type is specified in Table 6.1-1.

Table 6.1-1: R2D Message Type

| R2D Message Type value | R2D message name |
| --- | --- |
| 000 | Reserved |
| 001 | A-IoT Paging message |
| 010 | Access Occasion Trigger message |
| 011 | Random ID Response message |
| 100 | R2D Upper Layer Data Transfer message |
| 101 | Reserved |
| 110 | Reserved |
| 111 | Reserved |

The D2R message type is the set of A-IoT MAC messages that are sent from the device to the reader on the PDRCH. The value of D2R message type is specified in Table 6.1-2.

Table 6.1-2: D2R Message type

| D2R Message Type value | D2R message name |
| --- | --- |
| NA | Random ID message |
|  | D2R Upper Layer Data Transfer message |
|  | Reserved |
|  | Reserved |
|  | Reserved |

Editor’s Note:	Other message types are FFS. The message types may evolve based on functionality agreements.

Editor’s Note:	FFS whether we introduce D2R message type.  Discuss after looking at the overall MAC header design and space before deciding whether we introduce message type or reserved bits.

## 6.2	A-IoT MAC messages

### 6.2.1	R2D messages

#### 6.2.1.1	A-IoT Paging message

The fields in this message are defined as follows:

-	R2D Message Type: This field indicates the message type. See the Table 6.1-1.

-	RA Type: This field indicates CBRA or CFRA.

-	Indication of Paging ID Presence: This field indicates whether Paging ID and Length of Paging ID are present (when set to 1) or absent (when set to 0).

-	Length of Paging ID: This field indicates the Paging ID length information when Paging ID field is present.

-	Paging ID: xxx

-	Transaction ID: xxx

-	Number of Access Occasions: This field indicates the number of access occasions.

-	D2R Scheduling Info: This field indicates the physical layer parameters used for D2R scheduling.

Editor’s Note:	FFS if CFRA can omit the fields of transaction ID, Indication of Paging ID Present, Number of access occasions.

Editor’s Note:	FFS the length of transaction ID.

#### 6.2.1.2	Access Occasion Trigger message

Figure 6.2.1.2-1 shows the format of the Access Occasion Trigger message.

The field in this message is defined as follows:

-	R2D Message Type: This field indicates the message type. See the Table 6.1-1.

#### 6.2.1.3	Random ID Response message (Msg2 in CBRA)

Figure 6.2.1.3-1 shows the format of the Random ID Response message.

The fields in this message are defined as follows:

-	R2D Message Type: This field indicates the message type. See the Table 6.1-1.

-	Echoed Random ID: 16 bits

-	Assigned AS ID: This field provides the value of AS ID which is 16 bits.

-	D2R Scheduling Info: This field indicates the physical layer parameters used for D2R scheduling.

Editor’s Note:	FFS how to indicate the new assigned AS ID is present or not. FFS how to include multiple echoed random ID(s) and D2R Scheduling Info (if also multiple).

#### 6.2.1.4	R2D Upper Layer Data Transfer message

Figure 6.2.1.4-1 shows the format of the R2D Upper Layer Data Transfer message.

The fields in this message are defined as follows:

-	R2D Message Type: This field indicates the message type. See the Table 6.1-1.

-	AS ID: This field provides/indicates the value of AS ID.

-	Length: xxx

-	Data SDU: xxx

-	Received Data Size: This field is to indicate the number of bytes successfully received by the reader.

-	D2R Scheduling Info: This field indicates the physical layer parameters used for D2R scheduling.

Editor’s Note:	FFS whether offset zero is always included. FFS whether the reader always includes the command for retransmission of segments.

### 6.2.2	D2R messages

#### 6.2.2.1	Random ID message (Msg1 in CBRA)

Figure 6.2.2.1-1 shows the format of the Random ID message.

The field in this message is defined as follows:

-	Random ID: 16-bit random number

Figure 6.2.2.1-1 MAC PDU of Random ID message

#### 6.2.2.2	D2R Upper Layer Data Transfer message

Figure 6.2.2.2-1 shows the format of the D2R Upper Layer Data Transfer message.

The fields in this message are defined as follows:

Editor’s Note:	FFS whether for D2R we need message type field.

-	More Data Indication:

-	Length: xxx

Editor’s Note:	FFS how this is provided (i.e. SDU length field or padding length field).  The size of length field is FFS.

-	Data SDU: xxx

-	MAC Padding: This field includes padding bits.

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |