| 3GPP TR 38.741 V0.2.1 (2023-10) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Non-Terrestrial Networks (NTN) L-/S-band for NR; (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	6

4	Background	6

5	Regulations	7

5.1	FCC	7

5.1.1	47 CFR § 25.202	7

5.1.2	47 CFR § 25.216	7

5.2	ETSI	8

5.2.1	ETSI EN 301 441	8

6	NR	10

6.1	Band plan and system parameters	10

6.2	UE requirements	11

6.2.1	UE transmitter characteristics	11

6.2.1.1	Maximum output power	11

6.2.1.2	MPR/A-MPR	11

6.2.1.2.1	Maximum power reduction	11

6.2.1.2.2	Additional maximum power reduction	12

6.2.1.2.2.1	 Configuration cases	12

6.2.1.2.2.2	Power back-off results for ETSI	13

6.2.1.2.2.2.1	5MHz channel	13

6.2.1.2.2.2.2	10MHz channel	14

6.2.1.2.2.2.3	15MHz channel	15

6.2.1.2.2.3	 Power back-off results for FCC	15

6.2.1.2.2.3.1	5MHz channel	15

6.2.1.2.2.3.2	10MHz channel	16

6.2.1.2.2.3.3	15MHz channel	17

6.2.1.3	Emission requirements and NS values	18

6.2.1.4	UE co-existence requirements	18

6.2.2	UE receiver characteristics	18

6.2.2.1	Reference sensitivity	18

6.2.2.2	In-band blocking	18

6.2.2.3	Out-of-band blocking	19

6.3	SAN requirements	19

Annex <X> (informative): Change history	20

For definitive guidance on drafting 3GPP TSs and TRs, see 3GPP TS 21.801 supplemented by the 3GPP web page http://www.3gpp.org/specifications-groups/delegates-corner/writing-a-new-spec.

Ensure all blue guidance text is removed before submitting the TS/TR to the TSG for approval.

# Foreword

This clause is mandatory; do not alter the text in any way other than to choose between "Specification" and "Report".

This Technical Specification|Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In drafting the TS/TR, pay particular attention to the use of modal auxiliary verbs! TRs shall not contain any normative provisions.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

This clause is optional. If it exists, it shall be the second unnumbered clause.

# 1	Scope

The present document is a technical report for the work item of the NTN L-S-band.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Background

3GPP RAN#86 meeting approved the Rel-17 WI that aimed to enable support of the 5G/NR radio access technology for non-terrestrial satellite deployments. One of the WI objectives was to define and introduce the corresponding bands into the 3GPP specifications. The outcome of these discussions was two NTN bands – L-band and S-band – which were added as bands n255 and n256, respectively. At the same time there exist other satellite deployments, some of which use mixed L-/S-band pairing, whereupon the UL part is on the L-band, while the DL part is on the S-band. To limit the overall workload in Rel-17, it was decided by RAN WG4 to focus on L- and S-bands with the common understanding that other potential NTN bands should follow the "normal" process of submitting a new spectrum WI.

3GPP RAN#98 meeting agreed a new spectrum WI to add support for a new NTN band with the DL part on the S-band (2500MHz) and the UL part on the L-band (1600MHz).

# 5	Regulations

NOTE:	The purpose of this section is to collect information from existing regulatory documents concerning regulatory rules applicable to this NTN L-/S-band.

## 5.1	FCC

### 5.1.1	47 CFR § 25.202

NOTE: This section will contain the corresponding excerpts concerning frequencies, frequency tolerance, and emission limits.

According to paragraph (a)(4)(i), the following frequencies are available for use by the 1.6/2.4 GHz Mobile-Satellite Service:

-	1610-1626.5 MHz: User-to-Satellite Link

-	1613.8-1626.5 MHz: Satellite-to-User Link (secondary)

-	2483.5-2500 MHz: Satellite-to-User Link

According to paragraph (f) on emission limitations, except for SDARS terrestrial repeaters and as provided for in paragraph (i), the mean power of emissions shall be attenuated below the mean output power of the transmitter in accordance with the schedule set forth in paragraphs (f)(1) through (f)(4) of this section:

(1)	In any 4 kHz band, the center frequency of which is removed from the assigned frequency by more than 50 percent up to and including 100 percent of the authorized bandwidth: 25 dB;

(2) In any 4 kHz band, the center frequency of which is removed from the assigned frequency by more than 100 percent up to and including 250 percent of the authorized bandwidth: 35 dB;

(3) In any 4 kHz band, the center frequency of which is removed from the assigned frequency by more than 250 percent of the authorized bandwidth: An amount equal to 43 dB plus 10 times the logarithm (to the base 10) of the transmitter power in watts;

(4) In any event, when an emission outside of the authorized bandwidth causes harmful interference, the Commission may, at its discretion, require greater attenuation than specified in paragraphs (f) (1), (2) and (3) of this section.

### 5.1.2	47 CFR § 25.216

NOTE: This section will contain the corresponding excerpts concerning limits on emissions from mobile earth stations for protection of aeronautical radionavigation-satellite service.

(c) The e.i.r.p. density of emissions from mobile earth stations placed in service after July 21, 2002 with assigned uplink frequencies between 1610 MHz and 1660.5 MHz shall not exceed −70 dBW/MHz, averaged over any 2 millisecond active transmission interval, in the band 1559-1605 MHz. The e.i.r.p. of discrete emissions of less than 700 Hz bandwidth from such stations shall not exceed −80 dBW, averaged over any 2 millisecond active transmission interval, in the 1559-1605 MHz band.

## 5.2	ETSI

### 5.2.1	ETSI EN 301 441

NOTE: This section will contain the corresponding excerpts from ETSI EN 301 441.

Table 5.2.1-1: Mobile Satellite Service frequency bands

| MES | MSS frequency bands |
| --- | --- |
| Transmit | 1610 – 1626.5 MHz |
| Receive | 1613.8 – 1626.5 MHz |
| Receive | 2483.5 – 2500 MHz |

Table 5.2.1-2: Maximum unwanted emissions outside the band 1610 MHz to 1626.5 MHz and the band 1626.5 MHz to 1628.5MHz.

| Frequency (MHz) | Carrier-on |  |  |
| --- | --- | --- | --- |
|  | EIRP (dBW) | Measurement bandwidth | Measurement method |
| 0.1 to 30 | -66 | 10 kHz | Peak-hold |
| 30 to 1 000 | -66 | 100 kHz | Peak-hold |
| 1000 to 1559 | -60 | 1 MHz | Average |
| 1559 to 1580.42 | -70 | 1 MHz | Average |
| 1580.42 to 1605 | -70 | 1 MHz | Average |
| 1605 to 1610 | -70 to -10 | 1 MHz | Average |
| 1610 to 1626.5 | N/A | N/A | N/A |
| 1626.5 to 1628.5 | N/A | N/A | N/A |
| 1628.5 to 1631.5 | -60 | 30 kHz | Average |
| 1631.5 to 1636.5 | -60 | 100 kHz | Average |
| 1636.5 to 1646.5 | -60 | 300 kHz | Average |
| 1646.5 to 1666.5 | -60 | 1 MHz | Average |
| 1666.5 to 2200 | -60 | 3 MHz | Average |
| 2200 to 12750 | -60 | 3 MHz | Peak-hold |
| NOTE 1: In the sub-band 1 573,42 MHz to 1 580,42 MHz, the average measurement time is 20 ms. NOTE 2: Linearly interpolated in dBW vs. frequency offset. |  |  |  |

Table 5.2.1-3: Maximum unwanted emissions within the band 1610MHz to 1 626.5MHz and the band 1626.5 MHz to 1628.5MHz of MES operating such that the nominated bandwidth is entirely or partially contained in the frequency band 1618.25 MHz to 1626.5MHz.

| Frequency offset (kHz) | Carrier-on |  |  |
| --- | --- | --- | --- |
|  | EIRP (dBW) | Measurement bandwidth | Measurement method |
| 0 to 160 | -35 | 30 kHz | Average |
| 160 to 225 | -35 to -38.5 | 30 kHz | Average |
| 225 to 650 | -38.5 to -45 | 30 kHz | Average |
| 650 to 1365 | -45 | 30 kHz | Average |
| 1365 to 1800 | -53 to -56 | 30 kHz | Average |
| 1800 to 16500 | -56 | 30 kHz | Average |
| NOTE 1: Frequency offset is determined from: i) the nearest edge of the nominated bandwidth of the nominal carrier closest to the MSS system operating in another operational band within the band 1 610 MHz to 1 626,5 MHz. The frequency offset is measured in the direction of the adjacent MSS system; ii) the upper edge of the nominated bandwidth of the carrier under test for emissions within the band 1626.5MHz to 1628.5MHz. NOTE 2: The measurement bandwidth used may be 3 kHz if the unwanted EIRP limits are reduced correspondingly. NOTE 3: Linearly interpolated in dBW vs. frequency offset. |  |  |  |

Table 5.2.1-4: Maximum unwanted emissions within the band 1610MHz to 1626.5 MHz and the band 1626.5MHz to 1628.5MHz of MES operating such that the nominated bandwidth is entirely contained in the frequency band 1610MHz to 1618.25MHz.

| Frequency offset (kHz) | Carrier-on |  |  |
| --- | --- | --- | --- |
|  | EIRP (dBW) | Measurement bandwidth | Measurement method |
| 0 to 160 | -32 | 30 kHz | Average |
| 160 to 2300 | -32 to -56 | 30 kHz | Average |
| 2300 to 18500 | -56 | 30 kHz | Average |
| NOTE 1: Frequency offset is determined from: i) the nearest edge of the nominated bandwidth of the nominal carrier closest to the MSS system operating in another operational band within the band 1 610 to 1 626,5 MHz The frequency offset is measured in the direction of the adjacent MSS system; ii) the upper edge of the nominated bandwidth of the carrier under test for emissions within the band 1626.5 to 1628.5MHz. NOTE 2: The measurement bandwidth used may be 3 kHz if the unwanted EIRP limits are reduced correspondingly. NOTE 3: Linearly interpolated in dBW vs. frequency offset. |  |  |  |

Table 5.2.1-5: Maximum unwanted emissions of MES carriers within the operational band of CDMA carriers.

| Frequency offset (kHz) | Carrier-on |  |  |
| --- | --- | --- | --- |
|  | EIRP (dBW) | Measurement bandwidth | Measurement method |
| 0 to 70 | -6 to -20 | 30 kHz | Average |
| 70 to 600 | -20 to -28 | 30 kHz | Average |
| 600 to 2000 | -28 to -45 | 30 kHz | Average |
| 2000 to 5000 | -45 to -69 | 30 kHz | Average |
| 5000 to 16500 | -69 | 30 kHz | Average |
| NOTE 1: Frequency offset is determined from edge of nominated bandwidth. NOTE 2: Linearly interpolated in dBW vs. frequency offset. |  |  |  |

The frequency offset and relative power level of the adjacent signal compared to the wanted signal shall take the values given in table 5.2.1-6. The adjacent signal shall occupy the same bandwidth as the wanted signal where BW is the wanted signal occupied bandwidth. There shall be no more than 0.5dB degradation in the receiver signal to noise ratio under these conditions.

Table 5.2.1-6: Adjacent Channel frequency and power level

| Signal | Centre frequency offset from wanted signal | Power level relative to wanted signal |
| --- | --- | --- |
| Adjacent signal | Signal bandwidth | 12 dB |

Receiver blocking characteristics are to prevent high power signals outside the receive frequency band from blocking the reception of signals inside the receive frequency band. The blocking characteristic is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels, without this unwanted input signal causing a degradation of the performance of the receiver beyond a specified limit. Receiver blocking is specified for in-band signals. In-band signals are signals in the range:

BEL-10MHz to BEU +10 MHz, where BEL and BEU are the lower and upper edges of the operating band respectively.

The receiver performance degradation, in terms of signal to noise ratio, shall not exceed 1dB when the unwanted signal as specified in Table 5.2.1-7 is present.

Table 5.2.1-7: Test parameters for in-band blocking characteristics

| Interfering Signal | In-band Frequency Range (MHz) | Frequency offset from wanted carrier (MHz) | Level (dBm) |
| --- | --- | --- | --- |
| CW | BEL-10MHz to BEU +10 MHz | 5 | -40 |

# 6	NR

## 6.1	Band plan and system parameters

Table 6.1-1: NTN L-/S-band n254

| NTN satellite operating band | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n254 | 1610 – 1626.5 MHz | 2483.5 – 2500 MHz | FDD |
| NOTE: NTN satellite bands are numbered in descending order from n256. |  |  |  |

Table 6.1-2: NTN L-/S-band n254 channel bandwidths

| NTN satellite band | SCS kHz | UE Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 |  |
| n254 | 15 | 5 | 10 | 15 |  |
|  | 30 |  | 10 | 15 |  |
|  | 60 |  | 10 | 15 |  |

Table 6.1-3: NTN L-/S-band n254 applicable NR-ARFCN

| NTN satellite operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n254 | 100 | 322000 – <20> – 325300 | 496700 – <20> – 500000 |
| NOTE : The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. |  |  |  |

Table 6.1-4: NTN L-/S-band n254 applicable SS raster entries

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n254 | 15kHz | Case A | 6215 – <1> – 6244 |
|  | 30 kHz | Case C | 6218 – <1> – 6241 |
| NOTE : SS Block pattern is defined in clause 4.1 in 3GPP TS 38.213 [7]. |  |  |  |

Table 6.1-5: NTN L-/S-band asymmetric UL and DL channel bandwidth combinations

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) | Asymmetric channel bandwidth combination set |
| --- | --- | --- | --- |
| n254 | 5 | 10,15 | 0 |
|  | 10 | 15 | 0 |
| NOTE 1: The assignment of the paired UL and DL channels are subject to a TX-RX separation as specified in clause 5.4.4. NOTE 2: As indicated in TS38.306 [15], it is mandatory for UEs to support asymmetric channel BCS0 if there is an asymmetric BCS0 defined for the band. |  |  |  |

Table6.1-6: NTN L-/S-band UE TX-RX frequency separation

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n254 | 862 – 885 MHz |

## 6.2	UE requirements

### 6.2.1	UE transmitter characteristics

#### 6.2.1.1	Maximum output power

Table 6.2.1.1-1: NTN L-/S-band n254 Power Class

| NR satellite band | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| n254 | 23 | ±2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated |  |  |

#### 6.2.1.2	MPR/A-MPR

##### 6.2.1.2.1	Maximum power reduction

Maximum power reduction for the NTN L-/S-band is the same as for other NTN bands, such as L-band n255 and S-band n256.

Table 6.2.1.2.1-1: Maximum power reduction (MPR) for power class 3.

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.51 | ≤ 1.21 | ≤ 0.21 |
|  |  | ≤ 0.52 | ≤ 0.52 | 02 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.52 | 02 | 02 |
|  | QPSK | ≤ 1 |  | 0 |
|  | 16 QAM | ≤ 2 |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |
| NOTE 1: Applicable for UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and if the IE powerBoostPi2BPSK is set to 1 and 40 % or less slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. The reference power of 0 dB MPR is 26 dBm. NOTE 2: Applicable for UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79 with Pi/2 BPSK modulation and if the IE powerBoostPi2BPSK is set to 0 and if more than 40 % of slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. |  |  |  |  |

##### 6.2.1.2.2	Additional maximum power reduction

Maximum power reduction for the NTN L-/S-band is the same as for other NTN bands, such as L-band n255 and S-band n256.

###### 6.2.1.2.2.1		Configuration cases

For the power back-off simulations the following common parameters are assumed:

-	Tx power: PC3 +23dBm

-	LO placement: always in the centre of the carrier

-	Regulations: ETSI and FCC

The following channel sizes with centre frequencies are considered:

-	NR 5MHz channel at the following centre frequencies:

a)	Fc=1612.5MHz

b)	Fc=1615.5MHz

c)	Fc=1620.5MHz

d)	Fc=1624MHz

-	NR 10MHz channel at the following centre frequencies:

a)	Fc=1615MHz.

b)	Fc=1621.5MHz

-	NR 15MHz channel at the following centre frequencies:

a)	Fc=1617.5MHz

b)	Fc=1619MHz

Figure 6.2.1.2.2.1-1: Considered channels for A-MPR simulations.

###### 6.2.1.2.2.2	Power back-off results for ETSI

6.2.1.2.2.2.1	5MHz channel

Figure 6.2.1.2.2.2.1-1: Power back-off values for the 5MHz channel at Fc=1612.5MHz.

Figure 6.2.1.2.2.2.1-2: Power back-off values for the 5MHz channel at Fc=1615.5MHz.

Figure 6.2.1.2.2.2.1-3: Power back-off values for the 5MHz channel at Fc=1620.5MHz.

Figure 6.2.1.2.2.2.1-4: Power back-off values for the 5MHz channel at Fc=1624MHz.

6.2.1.2.2.2.2	10MHz channel

Figure 6.2.1.2.2.2.2-1: Power back-off values for the 10MHz channel at Fc=1615MHz.

Figure 6.2.1.2.2.2.2-2: Power back-off values for the 10MHz channel at Fc=1621.5MHz.

6.2.1.2.2.2.3	15MHz channel

Figure 6.2.1.2.2.2.3-1: Power back-off values for the 15MHz channel at Fc=1617.5MHz.

Figure 6.2.1.2.2.2.3-2: Power back-off values for the 15MHz channel at Fc=1619MHz.

###### 6.2.1.2.2.3		Power back-off results for FCC

6.2.1.2.2.3.1	5MHz channel

Figure 6.2.1.2.2.3.1-1: Power back-off values for the 5MHz channel at Fc=1612.5MHz.

Figure 6.2.1.2.2.3.1-2: Power back-off values for the 5MHz channel at Fc=1615.5MHz.

Figure 6.2.1.2.2.3.1-3: Power back-off values for the 5MHz channel at Fc=1620.5MHz.

Figure 6.2.1.2.2.3.1-4: Power back-off values for the 5MHz channel at Fc=1624MHz.

6.2.1.2.2.3.2	10MHz channel

Figure 6.2.1.2.2.3.2-1: Power back-off values for the 10MHz channel at Fc=1615MHz.

Figure 6.2.1.2.2.3.2-2: Power back-off values for the 10MHz channel at Fc=1621.5MHz.

6.2.1.2.2.3.3	15MHz channel

Figure 6.2.1.2.2.3.3-1: Power back-off values for the 15MHz channel at Fc=1617.5MHz.

Figure 6.2.1.2.2.3.3-2: Power back-off values for the 15MHz channel at Fc=1619MHz.

#### 6.2.1.3	Emission requirements and NS values

There are three additional NS values defined for this band as summarised in Table 6.2.1.3-1 below:

a)	NS_03N is for the FCC regulatory domain. Since FCC does not introduce any special in-band emission requirements for sub-ranges, same requirements will be applicable for the whole range and for all the channel sizes.

b)	ETSI regulations have different in-band emission masks depending on the operating sub-range, so two NS flags are needed to differentiate between them.

c)	NS_04N applicable range is limited to 1610-1618.25MHz according to the ETSI requirements. And since the total spectrum block size is 8.25MHz, it can accommodate only 5MHz standard NR channel, i.e. the applicable NR channel size is 5MHz.

d)	NS_04N is for the ETSI range 1618.25-1626.5MHz, which can also accommodate only 5MHz standard NR channel. However, ETSI document also clarifies that the same in-band emission requirements will apply if the channel is partially configured in that range. It means that any 10 and 15MHz channel configured in 1610-1626.5MHz range will inevitably fall into the 1618.25-1626.5MHz range and thus they will follow NS_04N requirements.

Table 6.2.1.3-1: Summary of the NS values

| Regulatory domain | NS value | Applicable range (MHz) | Applicable channel (MHz) |
| --- | --- | --- | --- |
| FCC | NS_03N | 1610-1626.5 | 5, 10, 15 |
| ETSI | NS_04N | 1610-1618.25 | 5 |
|  | NS_05N | 1618.25-1626.5 | 5 |
|  |  | 1610-1626.5 | 10, 15 |

Mapping between values of additionalSpectrumEmission and NS values is given in the Table 6.2.3.1-2 below.

Table 6.2.1.3-2: Mapping of network signalling label

| NR satellite band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n254 | NS_01 | NS_03N | NS_04N | NS_05N |  |  |  |  |

#### 6.2.1.4	UE co-existence requirements

Table 6.2.1.4-1: Requirements for spurious emissions for UE co-existence

| NR NTN satellite Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n254 | NR Band n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n24, n25, n26, n28, n29, n30, n34, n38, n39, n40, n41, n48, n50, n51, n53, n54, n65, n66, n67, n70, n71, n74, n75, n76, n77, n78, n85, n90, n91, n92, n93, n94, n100, n101, n105, n255, n256 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 31, 54, 72, 73, 87, 88, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79, n104 | FDL_low | - | FDL_high | -50 | 1 | 2 |

### 6.2.2	UE receiver characteristics

#### 6.2.2.1	Reference sensitivity

For band n254, band n53 REFSENS is re-used with additional 0.5dB insertion loss. The corresponding band n254 REFSENS values are presented in Table 6.2.2.1-1 below.

Table 6.2.2.1-1: NTN L-/S-band two antenna port reference sensitivity QPSK PREFSENS

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n254 | 15 | -99.5 | -96.3 | -94.5 |  |  |  |  |  |  |  |
|  | 30 |  | -96.6 | -94.6 |  |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 |  |  |  |  |  |  |  |

#### 6.2.2.2	In-band blocking

Table 6.2.2.2-1: NTN L-/S-band in-band blocking

| Operating Band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 |
| n254 | Finterferer (offset) | MHz | -BWChannel/2 – FIoffset, case 1 and BWChannel/2 + FIoffset, case 1 | ≤ -BWChannel/2 – FIoffset, case 2 and ≥ BWChannel/2 + FIoffset, case 2 |
|  | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -BWChannel/2 – FIoffset, case 1; b: BWChannel/2 + FIoffset, case 1 |  |  |  |  |

#### 6.2.2.3	Out-of-band blocking

For band n254, same out-of-band blocking requirements as for the band n53 are re-used, whereupon Range 3 exception is adjusted accordingly accounting for the fact that band n254 upper bound is at 2500MHz.

Table 6.2.2.3-1: Out of-band blocking requirements.

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n254 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE: Band n254 power level of the interferer (Pinterferer) for Range 3 shall be modified to -20 dBm for Finterferer > 2585 MHz and FInterferer < 2775 MHz. |  |  |  |  |  |

### 6.2.3	UE RRM requirements

There are no RRM specific requirements associated with this band.

## 6.3	SAN requirements

There are no SAN RF specific requirements associated with this band.

Annex <A> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-05 | RAN4#107 | R4-2309600 |  |  |  | TR skeleton | 0.0.0 |
| 2023-08 | RAN4#108 | R4-2314707 |  |  |  | Implementation of TPs agreed at RAN4#107 meeting | 0.1.0 |
| 2023-10 | RAN4#108bis |  |  |  |  | Implementation of agreements made at RAN4#108 meeting | 0.2.0 |
| 2023-10 | RAN4#108bis | R4-2311218 |  |  |  | TP containing power back-off values | 0.2.0 |
| 2023-10 | RAN4#108bis | R4-2315365 |  |  |  | TP containing summary of the NS values | 0.2.1 |
| 2023-10 | RAN4#108bis | R4-2315369 |  |  |  | TP containing summary of the RRM requirements | 0.2.1 |
| 2023-10 | RAN4#108bis | R4-2316134 |  |  |  | TP containing further clarifications for the REFSENS values | 0.2.1 |