| 3GPP TR 38.755 V0.3.0 (2025-05) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Networks; Study on NR FR1 DL fragmented carriers (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners’ Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles – Sophia Antipolis Valbonne – FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	7

4	Background	8

4.1	Objective	8

5	Study of methods for reducing the number of UE Rx chains	8

5.1	Scenarios of fragmented carriers	8

5.2	RF architectures	10

5.3	Reference architecture and RF performance	15

6	Study on power spectral density difference between carriers of co-located adjacent channel operators	18

7	Study on RF requirements for the inter-operator co-located BS scenario	22

7.1	RF requirements evaluation from company A	23

7.1.1	Analysis on Band n25	23

7.1.1.1	Case 1, 5 + 5 MHz	24

7.1.1.2	Case 2, 40 + 5 MHz	24

7.1.1a	Analysis on Band n25 with PCC SCC Swapping	25

7.1.2	Analysis on Band n2	28

7.1.3	Analysis on Band n3	29

7.1.4	Analysis on Band n7	30

7.1.5	Analysis on Band n26	31

7.1.6	Analysis on Band n41	32

7.1.6.1	Case 1, 10 + 10 MHz	32

7.1.6.2	Case 2, 40 + 40 MHz	33

7.1.7	Summary results for example bands	33

7.1.8 Discussion on in-gap requirements	34

7.2	RF requirements evaluation from company B	35

7.x Summary of evaluation results	39

8	Study of means for a UE to inform the network of appropriate CA configuration it can support	40

8.1	Indication of one Rx RF chain mode for DL fragmented CA	40

[8.1.1	Indication of fragmented CA alternative 1: Legacy band combination reporting scheme + Rx sharing capability indication]	40

[8.1.2	Indication of fragmented CA alternative 1a: Legacy band combination reporting scheme + Rx sharing capability and other related information]	40

[8.1.3	Indication of fragmented CA alternative 2: New notation for CA configurations with single Rx RF Chain]	41

8.1.4	Indication of fragmented CA alternative 2a: use the new bandwidth class conditioned on the frequency separation between two carriers	41

8.2	CA fallback and architecture switching	42

8.3	Triggering condition to enable ”One Rx RF chain”	42

8.3.1 Triggering condition proposal for option 1: Rely on existing channel quality reporting and procedures	42

8.3.2 Triggering condition proposal for option 2: UE measurement to determine and indicate to the network	42

9	Summary for NR FR1 DL fragment carriers study	42

Annex A (informative): Change history	43

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions “shall” and “shall not” are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions “must” and “must not” are not used as substitutes for “shall” and “shall not”. Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction “may not” is ambiguous and is not used in normative elements. The unambiguous constructions “might not” or “shall not” are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions “can” and “cannot” are not substitutes for “may” and “need not”.

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions “is” and “is not” do not indicate requirements.

1	Scope

The present document is a technical report for study of NR FR1 DL Fragmented Carriers.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: “Vocabulary for 3GPP Specifications”.

[2]	RP-241669, “New SID: Study on NR FR1 DL Fragmented Carriers”.

[3] 	3GPP TS 38.101-1: “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

[4] 	3GPP TS 38.101-2: “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”.

[5] 	3GPP TS 38.101-3: “NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”.

[6] 	R4-2413339 Discussion on methods for reducing the number of UE Rx chains for Fragmented Carriers, RAN4#112, Nokia

[7] 	R4-2415390 Discussion on methods for reducing the number of UE Rx RF chains for Fragmented Carriers, RAN4#112bis, Nokia

[8] 	RP-233374, Fragmented carriers in the DL, TELUS, Bell Mobility, T-Mobile USA, Telstra Limited, US Cellular Corporation, Nokia, Nokia Shanghai Bell

[9] 	R4-2411691 On general aspects of fragmented carriers, RAN4#112, Huawei

[10] 	RP-241269, On Fragmented Carriers in Rel-19, Apple

[11] 	RP-241361, Recommended WF on Fragmented Carriers, MediaTek

[12] 	R4-2417201 WF on NR FR1 DL fragmented carriers study, RAN4#112bis

[13] 	R4-2413031 On architecture options for fragmented spectrum reception, RAN4#112, Skyworks

[14] 	R4-2416138 Discussion on UE Rx chain architecture and network assumption, RAN4#112bis, MediaTek

[15] 	R4-2420519 WF on NR FR1 DL fragmented carriers study, RAN4#113

[16] 	R4-2415804 On methods for reducing the number of UE Rx chain, RAN4#112bis, Huawei

[17]	R4-2503479 TP to TR 38.755 on Clause 5, RAN4#114bis, CATT

[18]	R4-2503558 TP for TR38.755: Company C evaluation results, RAN4#114bis, Qualcomm

[19]	R4-2504452 TP on UE RF requirements impact for fragmented carriers, RAN4#114bis, Apple

[20]		R4-2504214 Further discussion of impacts on UE RF requirements and DL performance impacts, RAN4#114bis, MediaTek

[21]	R4-2504129 General aspects of Fragmented CA, RAN4#114bis, Nokia

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

A-MPR	Additional Maximum Power Reduction

BCS	Bandwidth Combination Set

CA	Carrier Aggregation

CC	Component Carrier

DC	Dual Connectivity

EIRP	Equivalent Isotropically Radiated Power

EN-DC	E-UTRA/NR DC

EVM	Error Vector Magnitude

FDM	Frequency Division Multiplexing

FR	Frequency Range

ENBW	The aggregated bandwidth of an E-UTRA sub-block and an adjacent NR sub-block

ITS	Intelligent Transportation System

ITU-R	Radiocommunication Sector of the International Telecommunication Union

MBW	Measurement bandwidth defined for the protected band

MPR	Allowed maximum power reduction

MSD	Maximum Sensitivity Degradation

MCG	Master Cell Group

NR	New Radio

NS	Network Signalling

NSA	Non-Standalone, a mode of operation where operation of an other radio is assisted with an other radio

OOB	Out-of-band

OOBE	Out-of-band emission

OTA	Over The Air

PRB	Physical Resource Block

RE	Resource Element

REFSENS	Reference Sensitivity

RF	Radio Frequency

Rx	Receiver

SCG	Secondary Cell Group

SCS	Subcarrier spacing

SEM	Spectrum Emission Mask

SUL	Supplementary uplink

TDM	Time Division Multiplex

Tx	Transmitter

UE	User Equipment

UL MIMO	Up Link Multiple Antenna transmission

ULSUP	Uplink sharing from UE perspective

4	Background

4.1	Objective

The objectives of the NR FR1 DL Fragmented Carriers SI are as follows:

-	Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains ) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering:

-	Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

-	Other requirements are not precluded, if identified

-	The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2];

-	Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4];

-	Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4]

-	Impacts on DL performance [RAN4] ;

-	Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2].

NOTE 1:	No RAN1 impact is foreseen

NOTE 2:	RAN2 work, if necessary, will be triggered by RAN4 LS

NOTE 3:	This study starts from single DL band. Sharing RF chain is not considered among inter-band DL carriers.

Whether and how to apply the conclusions to intra-band component of inter-band CA will be decided after the study on 2CC in a single DL band is completed.

5	Study of methods for reducing the number of UE Rx chains

5.1	Scenarios of fragmented carriers

An example to present the case of fragmented carriers is shown in the figure below from [6][7].

Figure 5-1: Example of fragmented carriers [8]

Fragmented carriers are not something completely new for RAN4, since it is essentially non-contiguous intra-band carrier aggregation (NC IB CA). However, proponents of the new SI have chosen the name as “Fragmented Carriers”, since this is how it is perceived mainly from an operator perspective. Adding a specific name also indicates a special way of handling non-contiguous intra-band carrier aggregation where UEs may support downlink reception of the fragmented spectrum block with a single “receive chain”, “receive path” or “receive branch”.

The conventional DL fragmented carriers reception containing more than one non-contiguous CC in the same band consuming multiple Rx RF chains may limit the UE CA capabilities as the maximum supported number of component carriers (CCs) in a CA combination is limited by the number and capabilities of the Rx RF chains as well as the baseband processor capabilities in the UEs.

From [9], a reference RF architecture of non-contiguous CA was initially introduced in LTE Rel-11, and captured in TR 36.823 as below.

Figure 5-2: Reference RF architecture of non-contiguous CA (Figure 6.2.3.1-1 of TR 36.823)

According to this reference RF architecture, the LNA is shared by two non-contiguous carriers, while the mixer, LPF and A/D are independent for each carrier. This reference RF architecture continues to be used in deriving the NR requirements for non-contiguous CA, i.e., the current RF receiving requirements in TS 38.101-1 for non-contiguous CA are based on this partially shared architecture.

5.2	RF architectures

The primary motivation of current study item is to reduce the number of UE Rx chains that are used to receive the fragmented carriers. To serve this motivation, different RF architectures were mentioned during early discussions, e.g. from separate RF chains per CC to shared RF chains.

Apple has in [10] provided what in their view is meant by a single Rx chain: “A single Rx chain consists of one main Rx and one diversity Rx for a UE with 2Rx”.

In [11] MediaTek shows a block diagram on the definition of a single Rx chain by applying Apple's definition in [13] of a Rx chain to the main and diversity branch in MediaTek’s block diagram, as shown in Fig. 5-3 and labelled as “Type 1” receiver.

It can be observed Type 1 UE receiver does not have the ability to suppress/attenuate in-gap interference via analogue filtering.

Type 1 – Single RX chain

| Single RX chain (consists of one main Rx and one diversity Rx) |  |
| --- | --- |
| Main receiver with RF carrier cantered between CCs | Diversity receiver with RF carrier centred between CCs |

Figure 5-3: Single RX chain as presented in [11], where DAC must mean ADCs, since this is at the receiver.

This is also illustrated in Fig. 5-4 with the main and diversity receiver branch.

Figure 5-4: Single Rx Chain for a 2Rx UE consisting of a main and diversity receiver branch (FDD).

It would be beneficial for further discussion to agree on what the underlying assumptions are, as a basis for a common understanding, to align terminologies which can help companies to ensure they discuss the same thing.

Similar to the previous discussion for 2Rx UEs, it becomes even more inconsistent what is meant by a Rx chain for 4Rx UEs when reviewing literature and previous contributions in RAN4. However, it seems the most predominant interpretation is to have 1 main receiver branch and 3 diversity receiver branches. Meaning, when using Fig. 5-4 as a reference, there will be an addition of two diversity receiver branches.

For the current UE implementation, the descriptions in [10] and [11] assume a separate UE RF chain per fragmented carrier which results in the limitations of the maximum number of supported CCs. For the current UE implementation, a separate UE RF chain per fragmented carrier is assumed.

This study item challenges the need for applying a separate UE Rx chain per fragment. In other words, the ambition is to reduce the number of needed Rx chains for fragmented carrier operation.

As previously mentioned, the concept of fragmented carries is in some way similar to non-contiguous intra-band carrier aggregation (NC IB CA). The reception at the UE of non-contiguous intra-band carrier aggregation in a system with a single Rx chain will be impacted by interference especially in the “gap” between the fragments of the carrier. The impact on UE requirements is further discussed in R4-2413340 while the following focuses on the UE receiver architectures.

The in-gap interference will, in a single analogue Rx chain system, pass un-attenuated as the UE does not have the ability to suppress/attenuate the in-gap interference via analogue filtering. Suggested solutions are shown in [9] where the UE, by utilizing additional Rx chains, can attend to this presence of an in-gap interferer. In [9] the single Rx chain system, as also shown in Fig. 5-3, is denoted Type 1 while two other multi-chain systems are denoted Type 2 and Type 3.

There are two ways that the signal can be treated by separate Rx chains. It may be split at the front-end components (eLNA), which is seen as Type 3, and it may also be split inside the transceiver, which is seen as Type 2.

Type 2 – Internal RX path split:

For Type 2 a full picture without the iLNA (internal LNA) split is discussed here. The UE has two Rx chains active, and this allows the UE to set the LO1 and LO2 at the respective centers of each CC, so that the subsequent filtering (channel filters) may attenuate also the in-gap interferer as shown here. In one example it considers non-collocation, where the in-gap interferer is seen as a “worst case” condition. In the collocated case, the wanted to interference ratio will be less.

Figure 5-5: Type 2 – Internal Rx path split. A single Rx chain after frontend filtering is marked with a dashed square.

If this UE is to transition to a single Rx chain state (Type 1), it will have to accept an in-gap interferer as shown in the next view of Type 2, that visualizes the “Extra Receiver chain” as toned-out colours:

Figure 5-6: Type 2 – Internal Rx path split. Unused Rx chain is toned-out. Type 2 becomes Type 1.

In this state, the UE is without analogue filtering to suppress the interferer, and RAN4 will have to discuss the relaxation of the receiver requirements for the UE to tolerate some level of in-gap interference presence. Since the requirement is based on the presence of an interfering signal, it is closer related to the ACS requirement than to the Reference sensitivity requirement for non-contiguous intra-band carrier aggregation, and a new set of requirements must capture the conditions and the relaxation for the UE state that uses a single Rx chain for the reception of the two fragmented carriers in the presence of an in-gap interferer.

This means that when a Type 2 implementation is using a subset of its Rx chains operating in fragmented carrier mode the new set of requirements will apply.

Type 3 – External Rx path split:

The second way that the signal can be treated by separate Rx chains is a split at the front-end components by multiple external LNAs (eLNA). This can either be within the main/diversity module, or at an extension from the main/diversity module that allows connection to another receive pin at the transceiver. For Type 3 the study would like to show a full picture and without discussion of detail whether front-end modules splits internally at the eLNA or if there’s a separate front-end module that accompanies the main/diversity modules to extend the connection possibilities towards the transceiver(s). Some solutions also have 2 transceivers that allow split of reception in NC IB CA between the two transceivers. The full system view is that the UE has two Rx chains active, and this allows the UE to set the LO1 and LO2 at the respective centers of each CC, so that the following filtering (channel filters) may attenuate also the in-gap interferer as shown here.

Figure 5-7: Type 3 – External Rx path split. A single Rx chain after frontend filtering is marked with a dashed square.

Details on the differences between the Type 2 and Type 3 UE architectures may be found in [10], where there is a split at the eLNA (external transceiver LNA) towards two iLNA (internal transceiver LNA) (Type2) and a split only at the DACs (Edit: it should mean ADCs, since this is at the receiver) for CC0 and CC1 (Type 3). However, it is not explaining how the UE actually may receive the two carriers individually.

To extend the understanding of the different choices for transceiver implementation, it would have been chosen to follow the principal presented by other companies and also split into three different types with emphasis on Type 2 and Type 3 in the following. This as Type 1 is the hardware state it is wanted to free up hardware resources i.e. Rx chains as described in the introduction.

It can be noted that there is also the possibility to suppress some of the in-gap interference in the digital domain. However, this does not solve the dynamic range issues given by the near-far problem. It can mitigate some of the adjacent channel interference but only when within proper dynamic range provided from the analog and mixed signal Rx chain. Hence, the focus of this work is on solutions related to the analog Rx path. Even though there may be a possibility to suppress some of the in-gap interference in the digital domain, the objective of the SI is to free up Rx resources hence the focus is on the analog Rx chains.

If this UE is to transition to a single Rx chain state (Type 1) it will have to tolerate an in-gap interferer as shown in the next view of Type 3, that visualizes the “Extra Receiver chain” as toned-out colours:

Figure 5-8: Type 3 – External Rx path split. Unused Rx chain is toned-out. Type 3 becomes Type 1.

In this state, the UE is in the same conditions as in Type 2, bringing both Type 3 and Type 2 into the state of Type 1 single Rx chain reception, so the relaxation of the requirements is the same for Type 2 and Type 3, and the same set of requirements can apply regardless of how the UE supports non-contiguous intra-band carrier aggregation.

As presented previously it has been shown that with both a Type 2 and a Type 3 transceiver implementation it is possible to separate the Rx chains (i.e. operate as Type 1). When separating or freeing up Rx chains in the Type 2 and Type implementations it is possible to apply these now idle Rx chains for reception of other carries if new RAN4 requirements for this type of operation is introduced to the specification.

5.3	Reference architecture and RF performance

In the RAN4#112bis meeting, it was agreed the reference architecture is based on fully shared RF chain [12].

The terminology of “One Rx RF Chain” is from single antenna to the end of ADC (before digital BB filter) with a single down-converter

-	To reduce the number of Rx RF chains, a fully shared RF chain (one Rx RF chain) is the reference architecture for fragmented carriers

Figure 5-9: Reference architecture of one Rx RF chain

-	A partially shared RF chain architecture is the reference architecture for existing DL NCCA requirements (refer to TR 36.823)

There are also some discussion on baseband assumptions i.e., shared/separated BB filter that may relate to rejection ability toward unwanted signal for the study as well as “single FFT” or “separate FFT” assumption evaluation.

Referring to [13], there may be different digital BB chain approaches:

o	Separate digital BB chains per CC after digital down conversion which would enable a dedicated digital BB filter and thus good ACS.

o	Common digital BB chain assuming that CCs are SCS aligned such that a shared FFT/digital filter are used. In this case it may be difficult to maintain good ACS and blocking performance, especially in the gap.

•	Note that if a single FFT is used to receive two non-contiguous CCs/cluster that are SCS aligned, if the CCs are 15kHz SCS the maximum BW is 50MHz and if 30kHz it is 100MHz.

Figure 5-9 illustrates some of the above aspects and terminology. The CCs in the illustration are the co-located operator non-contiguous CCs/clusters to be received.

Figure 5-10 illustration of shared Analog path and shared/separate digital BB paths

For the baseband signal processing, referring to[14], separate FFT is legacy implement to handle DL NCCA carriers since it provides better rejection on interference and unwanted signal. Single FFT that can receive both NCCA carriers including in-gap interference (with poor rejection) at a time. There are some constraint for single FFT. The channel raster of PCC and SCC shall align to orthogonal grid that can avoid unnecessary ICI after FFT. This would need to be guaranteed by network scheduling. And due to the max FFT size is 4096, for the fragment carriers that span larger than 50MHz, only 30KHz or 60KHz SCS would be applicable for the case.

With above assumptions, the evaluated RF performance when there exists in-gap interference applying single FFT together is shown below. The fc of interference was assumed in the middle of two FFT grid as worst-case simulation.

Figure 5-11: SCC: 5M, PCC: 5M + ACS single FFT simulated spectrum plot

Figure 5-12: SCC: 5M, PCC: 5M + IBB1 single FFT simulated spectrum plot

Figure 5-13: SCC: 5M, PCC: 5M + IBB2 single FFT simulated spectrum plot

It can be seen from simulated results, the spectrum of interference was spread out due to allocated on non-orthogonal FFT grid. UE would be able to pass ACS1/2 and IBB1 requirements without relaxation but would fail on IBB2. And due to wide frequency span for a single FFT, the baseband power consumption could be higher. It is expected single FFT may have worse rejection ratio to interference signal and this is not common baseband implementation for DL NCCA carriers.

If UE apply separate FFT baseband, it is foreseen the digital filter can provide better rejection on the in-gap interference signal so that performance of wanted CCs would not be degraded except when the interference falls in the Rx image range of wanted signal. Separate FFT for DL NCCA carriers has been widely used for modern modem design.

It has been agreed in [15] single FFT or separate FFT can be left to UE implementation, and however the condition of single FFT compared to separate FFT can be captured in the technical report.

6	Study on power spectral density difference between carriers of co-located adjacent channel operators

It is also intended to study on power spectral density difference between carriers of collocated adjacent channel inter-operators impact on the UE in the objective of current SI. The PSD difference between operators is for further study. Here evaluation results contributed by different companies are captured as below.

Evaluation from company A:

Referring to [16], according the discussion in RAN4#112, it’s very difficult to ensure different operators are always co-located. Even in some cases the operators share the same site, it’s very common that the infrastructures are not shared. As a result, the coverage of different operators is not fully aligned. Below Figure 6-1 shows a possible multi-operator deployment case.

Figure 6-1: Multi-operator deployment case

In this example, the UE is at the edge of its serving cell, while almost in the center of the neighbouring operator cell. Taking the BS antenna pattern from TR 38.769 (fixed beam direction) as the assumption for both cells, the UE’s maximum receiving RSRP difference is equal to the antenna gain difference between horizontal angle = 0 degree and +/-60 degree. The Figure 8 shows the RSRP difference could achieve 10dB in this case.

Figure 6-2: Delta RSRP Multi-operator deployment case

If the deployment includes both macro cell and micro cell, the possible RSRP difference will increase. Also the fast fading will bring more fluctuation to the RSRP difference. For the purpose of study, it can be considered a higher level RSRP difference, e.g. 20dB.

Evaluation from company B:

Referring to [14], some system level simulation results which follows coexistence frame work considering large-scale parameters were also provided. The environment and configuration assumptions are as following bullets:

Co-site case

Same BW/Power/ANT # of 2 operators at n25 (1.9GHz)

10 MHz(wanted)+10 MHz(unwanted) BW

Care interference from the same site.

Log RSRP difference between 2 operators à CDF

The detailed parameters are listed in the table below:

Table 6-1: Simulation parameters of PSD difference from UE reception point of view

| Parameter | setting |
| --- | --- |
| Scenario | Dense Urban |
| Channel model | UMa |
| ISD | 200m |
| Number of operators | 2 |
| Number of sites | 7/operator, each with 3 sectors |
| Number of UEs | 10/sectors |
| Carrier frequency | 1.9GHz |
| Channel BW | 10MHz |
| Antenna pattern | AAS is assumed (same as that in coexistence study) |
| BS antenna configuration | 32 ANT# : (M,N,P) = (4,4,2) 128 ANT# : (M,N,P) = (8,8,2) |
| Note: The simulation follows coexistence framework, which considers large-scale parameters only. |  |

Figure 6-3: Antenna patterns for the simulation

Figure 6-4: Simulated PSD difference at UE receiving antenna

From simulation result, it can be seen there’s no “interference level larger than wanted CC case” if the BW of both wanted CCs and unwanted CC(=Wgap) are same. The maximum ratio of unwanted CC level to wanted CC level is 0dB in this case. Other different BW/Wgap configurations can be scalable estimated that existing requirements at in-gap ACS1/ACS2/IBB1/IBB2/NBB can fulfil the need for coexistence. On the other hand, collocation inter-operator assumption might be too much simplified and different with real deployment. Some company propose to keep same ACS/IBB requirements and the proposal can be further considered for non-collocated BS cases. And even applying the new UE capability that RF parts can support higher order CA configurations with more CCs, the baseband processing capability may need to extend to handle higher order CA combos including more DL carriers as well as more MIMO-layer processing capability.

Evaluation from company C:

This evaluation is based on system level simulation procedure long established in 3GPP, according to the assumptions listed in this section. The result metric provided is the CDF of the difference in RSRP between the serving operator BS and the strongest interfering BS from the other operator. We have varied some large scale deployment parameters (Antenna Panel, Network Shift).

NW shift of 50% and 100% are computed respectively using ISD*sqrt(3)/3/2 and ISD*sqrt(3)/3.

Table 6-2: System Level Simulation Parameters for Company C evaluation

| Parameter | Value | Unity of Measure |
| --- | --- | --- |
| Number of Operators | 2 | - |
| Deployment Scenario | Dense Urban | - |
| Channel Model | UMa (38.901) | - |
| Building Entry Loss Model | P.2109 | - |
| Frequency | 1.9 | GHz |
| Channel Bandwidth | 10 | MHz |
| ISD | 750 | meters |
| Antenna Model | Sector Antenna, 32Tx AAS (4,4,2) 128Tx AAS (8,8,2) | - |
| Active UEs per sector | 1 | - |
| Number of sites/sectors | 19 sites with 3 sectors each | - |
| Network shift across different operators | Co-site 50% 100% | - |
| Sector orientation across different operators | Same | - |

Figure 6-5: Difference between Own BS RSRP and Strongest Interfering BS RSRP [ISD = 750m]

The Figure 6-5 is in agreement with the Figure 6-4 but more broader view is taken[18].

7	Study on RF requirements for the inter-operator co-located BS scenario

It has been agreed baseline UE architecture is one Rx RF chain from single antenna to the end of ADC with a single down-converter. The evaluation assumptions were also agreed as summarized and re-organized bellow[4][5]:

Example bands and CBW/BCS assumptions:

n2/n25, n3, n7, n66, n41, n26

Prioritize the frequency bands of ≤2700MHz

The example band(s) should already have NC DLCA specified

Don’t consider 3MHz channel bandwidth for fragmented carriers

Both the symmetric CC scenario and the asymmetric CC scenario should be considered in the FC SI, and companies are encouraged, but not limited, to study the CC scenarios shared by the operators

Assumptions for collocated adjacent channel inter-operator:

To evaluate acceptable [PSD] difference from UE side between wanted signal and un-wanted signal based on the deployment where BSs from different operators are collocated but their radio units (RUs) are not shared.

The [PSD] difference means the difference between unwanted signal and the wanted carrier with the lower PSD

Start evaluation with wanted signal level currently existing ACS requirements as baseline and to see the performance difference

Assumptions for Tx:

Re-use same assumptions for NR PC3 PA i.e., ACLR=30dB, with MPR=1dB, Full RB allocation. Tx LO leakage and image rejection ratio are 28dB

One UL carrier is prioritized

UL power class PC3 is prioritized. Both PC3 and PC2 should be considered

No relaxation on any Tx RF requirements should be assumed in this study item

Assumptions for Rx:

One Rx RF chain (fully shared Rx) is assumed

Reuse the Rx chain image rejection, which is 25dB, as the starting point

Baseband assumptions:

Consider only same SCS for both CCs for one Rx RF chain architecture in this SI

15KHz SCS is used as the starting point

Single FFT or separate FFT can be left to UE implementation, and however the condition of single FFT compared to separate FFT can be captured in the TR

Assumptions for test conditions:

The configuration defined in Table 7.3A.2.2-1 and Table 7.3A.2.2-2 of TS 38.101-1 could be re-used

Evaluation may consider impairment factors such as ACLR, phase noise or IMDx of aggressor ability, duplexer isolation…etc

The minimum Wgap for ACS, narrow band blocking and in-band blocking verification should meet

Wgap ≥ 2∙|FInterferer (offset),j|  – BWChannel(j)

It is the condition to apply the requirements

[Adjusted requirements for fragmented carriers, if any, shall be separated from the existing requirements]

The requirements shall be discussed and decided case by case manner per operators’ request on target band and associated configuration

RAN4 also agreed a common table format to demonstrate evaluation parameters and results. Companies can provide evaluation results for example bands in bellow:

7.1	RF requirements evaluation from company A

7.1.1	Analysis on Band n25

If the Rx chain is shared by the two non-contiguous CCs, (fully-shared Rx chain), the channel selection filter would need to be adjusted to wider bandwidth to accommodate the two DL carriers including Wgap between the two CCs. The Tx rejection of shared Rx chain would be much worse. Not only SCC would still impacted by self-band UL Tx, PCC may also be impacted due to worse filter rejection ratio toward UL Tx transmission. It is foreseen the DL AGC would need to be adjusted to tolerate larger Tx leakage portion thus the gain setting would become lower that brings noise level raised up for both PCC and SCC. Another potential degradation may happen on in-gap ACS/IBB/NBB performance, these requirements would also need to be evaluated.

Figure 7.1.1-1: Rx requirements to be evaluated

Figure 7.1.1-2: Illustration on the two BCS for DL NCCA test

The existing requirements of n25(2A) is copied below:

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | SCC ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n25(2A) 9 | 15/15 | 5MHz + 5MHz | Wgap = 55.0 | 105 | 5.0 | FDD |
|  |  |  | Wgap = 30.0 | 25 | 0.0 |  |
| CA_n25(2A) 10 | 15/15 | 40MHz + 5MHz | Wgap = 20.0 | 40 (RBstart = 176) | [24.6] 8 | FDD |

7.1.1.1	Case 1, 5 + 5 MHz

Based on the agreed assumptions and reference table format, we put our evaluation parameters in the table 1 with proper change. We think assuming 0dBFS for the ADC input target level may not be a proper setting. In most cases, there would be PAPR, radio signal fluctuation and fading effect for the input signal. So we would suggest not to consider this case. The table is modified as below:

Table 7.1.1-1: Link budget of Self-interference for 5+5 MHz on band n25

|  | Tx leakage | Wanted Carrier | Wanted Carrier | Noise floor | ADC noise (11 bit): -68.0dBFs |
| --- | --- | --- | --- | --- | --- |
|  |  | (PCC) | (SCC) |  | (6.02N+1.76) |
|  |  |  |  |  | Tx leakage mapping to |
|  |  |  |  |  | -12dBFS (PAPR is considered) |
| PA out | 27 | / | / | / | / |
| Duplexer isolation | 50 | / | / | / | / |
| Rx Antenna input | / | -96.5 | -91.5 | / | / |
| FE loss | / | 5 | 5 | / | / |
| Duplexer output | -23 | -101.5 | -96.5 | / | / |
| LPF attenuation | 6.1 | 1 | 1 | / | / |
| LPF output (dBm) | -29.1 | -102.5 | -97.5 | / | / |
| Noise floor for basic REFSENS | / | / | / | -101.5 | / |
| NF increase due to AGC adjustment | / | / | / | 13.1 | / |
| Noise floor | / | / | / | -88.4 | / |
| In-band noise caused by Tx leakage (Derived based on MSD and -1dB SNR) | / | / | -96.5 | / | / |
| ADC output (dBm) | / | / | / | / | -96.2 |
| SNR after ADC (dB) | / | / | / | / | -13.8(PCC) |
|  |  |  |  |  | -14.3(SCC) |

7.1.1.2	Case 2, 40 + 5 MHz

The REFSENS of 40MHz is not linear scaling from 5MHz in existing specs. So we had calculated excessive Tx leakage level fall in the DL receiving channel in. The evaluation parameters and results are demonstrated in below table.

Table 7.1.1-2: Link budget of Self-interference for 40+5 MHz on band n25

|  | Tx leakage | Wanted Carrier | Wanted Carrier | Noise floor(PCC) | Noise floor(SCC) | ADC noise (11 bit): -68.0dBFs |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | (PCC) | (SCC) |  |  | (6.02N+1.76) |
|  |  |  |  |  |  | Tx leakage mapping to |
|  |  |  |  |  |  | -12dBFS (PAPR is considered) |
| PA out | 27 | / | / | / | / | / |
| Duplexer isolation | 50 | / | / | / | / | / |
| Rx Antenna input | / | -79.5 | -71.9 | / | / | / |
| FE loss | / | 5 | 5 | / | / | / |
| Duplexer output | -23 | -84.5 | -76.9 | / | / | / |
| LPF attenuation | 6.1 | 1 | 1 | / | / | / |
| LPF output (dBm) | -29.1 | -85.5 | -77.9 | / | / | / |
| Noise floor for basic REFSENS | / | / | / | -92.5 | -101.5 | / |
| NF increase due to AGC adjustment | / | / | / | 13.1 | 13.1 | / |
| Noise floor | / | / | / | -79.4 | -88.4 | / |
| In-band noise caused by Tx leakage (Derived based on MSD and -1dB SNR) | / | / | -76.9 | / | / | / |
| ADC output (dBm) | / | / | / | / | / | -87.2 |
| SNR after ADC (dB) | / | / | / | / | / | -6.5(PCC) |
|  |  |  |  |  | / | -24.9(SCC) |

7.1.1a	Analysis on Band n25 with PCC SCC Swapping

In the last RAN4 meeting, concerns on PCC performance degradation were raised and an alternative approach was also provided. In the network scheduling control, PCC SCC can be swapped to make Tx-Rx frequency gap larger to reduce performance degradation. The spectrum plot is illustrated below Figure 1. With the swapping, it is expected the Rx performance degradation can be much reduced.

Figure 7.1.1a-1. Illustration on PCC/SCC swapping for the two cases for DL NCCA test

The evaluation results are as following.

Case 1, 5 + 5 MHz

Based on the agreed assumptions and reference table format,

Evaluation results on PCC/SCC swapping are in the updated table below:

Table 7.1.1a-1: Link budget of Self-interference for 5+5 MHz on band n25

|  | Tx leakage | Wanted Carrier | Wanted Carrier | Noise floor | ADC noise (11 bit): -68.0dBFs |
| --- | --- | --- | --- | --- | --- |
|  |  | (PCC) | (SCC) |  | (6.02N+1.76) |
|  |  |  |  |  | Tx leakage mapping to |
|  |  |  |  |  | -12dBFS (PAPR is considered) |
| PA out | 27 | / | / | / | / |
| Duplexer isolation | 50 | / | / | / | / |
| Rx Antenna input | / | -96.5 | -96.5 | / | / |
| FE loss | / | 5 | 5 | / | / |
| Duplexer output | -23 | -101.5 | -101.5 | / | / |
| LPF attenuation | 22.2 | 1 | 1 | / | / |
| LPF output (dBm) | -45.2 | -102.5 | -102.5 | / | / |
| Noise floor for basic REFSENS | / | / | / | -101.5 | / |
| NF increase due to AGC adjustment | / | / | / | 4 | / |
| Noise floor | / | / | / | -97.5 | / |
| In-band noise caused by Tx leakage (Derived based on MSD and -1dB SNR) | / | / | -101.5 | / | / |
| ADC output (dBm) | / | / | / | / | -112.3 |
| SNR after ADC (dB) | / | / | / | / | -4.0(PCC) |
|  |  |  |  |  | -3.4(SCC) |

Case 2, 40 + 5 MHz

The REFSENS of 40MHz is not linear scaling from 5MHz in existing specs. Excessive Tx leakage level fall in the DL receiving channel is calculated in the table. The evaluation parameters and results are demonstrated in below table.

Table 7.1.1a-2: Link budget of Self-interference for 40+5 MHz on band n25

|  | Tx leakage | Wanted Carrier | Wanted Carrier | Noise floor(PCC) | Noise floor(SCC) | ADC noise (11 bit): -68.0dBFs |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | (PCC) | (SCC) |  |  | (6.02N+1.76) |
|  |  |  |  |  |  | Tx leakage mapping to |
|  |  |  |  |  |  | -12dBFS (PAPR is considered) |
| PA out | 27 | / | / | / | / | / |
| Duplexer isolation | 50 | / | / | / | / | / |
| Rx Antenna input | / | -79.5 | -89.3 | / | / | / |
| FE loss | / | 5 | 5 | / | / | / |
| Duplexer output | -23 | -84.5 | -94.3 | / | / | / |
| LPF attenuation | 11.1 | 1 | 1 | / | / | / |
| LPF output (dBm) | -34.1 | -85.5 | -95.3 | / | / | / |
| Noise floor for basic REFSENS | / | / | / | -92.5 | -101.5 | / |
| NF increase due to AGC adjustment | / | / | / | 7.2 | 7.2 | / |
| Noise floor | / | / | / | -85.3 | -94.3 | / |
| In-band noise caused by Tx leakage (Derived based on MSD and -1dB SNR) | / | / | -94.3 | / | / | / |
| ADC output (dBm) | / | / | / | / | / | -92.2 |
| SNR after ADC (dB) | / | / | / | / | / | -2.2(PCC) |
|  |  |  |  |  | / | -8(SCC) |

Summary results for band n25 with PCC/SCC swapping

Evaluated results are summarized in the table below:

Table 7.1.1a-3: Proposed ΔRIBNC for example band n25 on Rx RF chain mode with P/S cell swapping

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | PCC ΔRIBNC (dB) | SCC ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n25(2A) | 15/15 | 5MHz + 5MHz | Wgap = 55.0 | 10 | 13.8 4.0 | 14.3 3.4 | FDD |
|  |  |  | Wgap = [10 20] | 25 | 0 | 0 |  |
| CA_n25(2A) | 15/15 | 40MHz + 5MHz | Wgap = 20.0 | 40 (RBstart = 176) | 6.5 2.2 | 24.9 8 | FDD |

Observation1: It can be observed if P/S cell are swapped, the REFSENS degradation can be lower to ~4dB for 5+5MHz case. For 40+5MHz case, the degradation can be lower to 2~8dB

7.1.2	Analysis on Band n2

The frequency span of band n2 is 5MHz smaller than band n25. So the gap between Tx and Rx can be slightly larger so that the evaluated result may be a bit better (less ΔRIBNC). Following common assumptions, the evaluation result are demonstrated in below tables.

Table 7.1.2-1: Link budget of Self-interference for 5+5 MHz on band n2

|  | Tx leakage | Wanted Carrier | Wanted Carrier | Noise floor | ADC noise (11 bit): -68.0dBFs |
| --- | --- | --- | --- | --- | --- |
|  |  | (PCC) | (SCC) |  | (6.02N+1.76) |
|  |  |  |  |  | Tx leakage mapping to |
|  |  |  |  |  | -12dBFS (PAPR is considered) |
| PA out | 27 | / | / | / | / |
| Duplexer isolation | 50 | / | / | / | / |
| Rx Antenna input | / | -98 | -93 | / | / |
| FE loss | / | 5 | 5 | / | / |
| Duplexer output | -23 | -103 | -98 | / | / |
| LPF attenuation | 11.8 | 1 | 1 | / | / |
| LPF output (dBm) | -34.8 | -104 | -99 | / | / |
| Noise floor for basic REFSENS | / | / | / | -103 | / |
| NF increase due to AGC adjustment | / | / | / | 11.6 | / |
| Noise floor | / | / | / | -91.4 | / |
| In-band noise caused by Tx leakage (Derived based on MSD and -1dB SNR) | / | / | -98 | / | / |
| ADC output (dBm) | / | / | / | / | -101.6 |
| SNR after ADC (dB) | / | / | / | / | -12(PCC) |
|  |  |  |  |  | -12.8(SCC) |

7.1.3	Analysis on Band n3

Band n3 is a similar case with band n2, 5+5 MHz configuration. The evaluated results are demonstrated in below tables.

Table 7.1.3-1: Link budget of Self-interference for 5+5 MHz on band n3

|  | Tx leakage | Wanted Carrier | Wanted Carrier | Noise floor | ADC noise (11 bit): -68.0dBFs |
| --- | --- | --- | --- | --- | --- |
|  |  | (PCC) | (SCC) |  | (6.02N+1.76) |
|  |  |  |  |  | Tx leakage mapping to |
|  |  |  |  |  | -12dBFS (PAPR is considered) |
| PA out | 27 | / | / | / | / |
| Duplexer isolation | 50 | / | / | / | / |
| Rx Antenna input | / | -97 | -92.3 | / | / |
| FE loss | / | 5 | 5 | / | / |
| Duplexer output | -23 | -102 | -97.3 | / | / |
| LPF attenuation | 9 | 1 | 1 | / | / |
| LPF output (dBm) | -32 | -103 | -98.3 | / | / |
| Noise floor for basic REFSENS | / | / | / | -102 | / |
| NF increase due to AGC adjustment | / | / | / | 11.7 | / |
| Noise floor | / | / | / | -90.3 | / |
| In-band noise caused by Tx leakage (Derived based on MSD and -1dB SNR) | / | / | -97.3 | / | / |
| ADC output (dBm) | / | / | / | / | -99.8 |
| SNR after ADC (dB) | / | / | / | / | -12.2(PCC) |
|  |  |  |  |  | -12.9(SCC) |

7.1.4	Analysis on Band n7

The band n7 evaluated results are demonstrated in below tables.

Table 7.1.4-1: Link budget of Self-interference for 10+5 MHz on band n7

|  | Tx leakage | Wanted Carrier | Wanted Carrier | Noise floor(PCC) | Noise floor(SCC) | ADC noise (11 bit): -68.0dBFs |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | (PCC) | (SCC) |  |  | (6.02N+1.76) |
|  |  |  |  |  |  | Tx leakage mapping to |
|  |  |  |  |  |  | -12dBFS (PAPR is considered) |
| PA out | 27 | / | / | / | / | / |
| Duplexer isolation | 50 | / | / | / | / | / |
| Rx Antenna input | / | -94.8 | -98 | / | / | / |
| FE loss | / | 5 | 5 | / | / | / |
| Duplexer output | -23 | -99.8 | -103 | / | / | / |
| LPF attenuation | 17.2 | 1 | 1 | / | / | / |
| LPF output (dBm) | -40.2 | -100.8 | -104 | / | / | / |
| Noise floor for basic REFSENS | / | / | / | -99.8 | -103 | / |
| NF increase due to AGC adjustment | / | / | / | 6.1 | 6.1 | / |
| Noise floor | / | / | / | -93.7 | -96.9 | / |
| In-band noise caused by Tx leakage (Derived based on MSD and -1dB SNR) | / | / | -103 | / | / | / |
| ADC output (dBm) | / | / | / | / | / | -104.7 |
| SNR after ADC (dB) | / | / | / | / | / | -6.4(PCC) |
|  |  |  |  |  | / | -6.4(SCC) |

7.1.5	Analysis on Band n26

The band n26 evaluated results are demonstrated in below tables.

Table 7.1.5-1: Link budget of Self-interference for 15+10 MHz on band n26

|  | Tx leakage | Wanted Carrier | Wanted Carrier | Noise floor(PCC) | Noise floor(SCC) | ADC noise (11 bit): -68.0dBFs |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | (PCC) | (SCC) |  |  | (6.02N+1.76) |
|  |  |  |  |  |  | Tx leakage mapping to |
|  |  |  |  |  |  | -12dBFS (PAPR is considered) |
| PA out | 27 | / | / | / | / | / |
| Duplexer isolation | 50 | / | / | / | / | / |
| Rx Antenna input | / | -92.7 | -69.3 | / | / | / |
| FE loss | / | 5 | 5 | / | / | / |
| Duplexer output | -23 | -97.7 | -74.3 | / | / | / |
| LPF attenuation | 9.7 | 1 | 1 | / | / | / |
| LPF output (dBm) | -32.7 | -98.7 | -75.3 | / | / | / |
| Noise floor for basic REFSENS | / | / | / | -97.7 | -93.5 | / |
| NF increase due to AGC adjustment | / | / | / | 13.3 | 13.3 | / |
| Noise floor | / | / | / | -84.4 | -80.2 | / |
| In-band noise caused by Tx leakage (Derived based on MSD and -1dB SNR) | / | / | -74.3 | / | / | / |
| ADC output (dBm) | / | / | / | / | / | -92.4 |
| SNR after ADC (dB) | / | / | / | / | / | -13.9(PCC) |
|  |  |  |  |  | / | -26.2(SCC) |

7.1.6	Analysis on Band n41

The main difference of evaluation results between TDD band and FDD band is due to there’s no Tx leakage fall into DL receiving channel since there’s no UL transmission during receiving mode.

7.1.6.1	Case 1, 10 + 10 MHz

Table 7.1.6-1 Link budget of Self-interference for 10+10 MHz on band n41

|  | Tx leakage | Wanted Carrier | Wanted Carrier | Noise floor | ADC noise (11 bit): -68.0dBFs |
| --- | --- | --- | --- | --- | --- |
|  |  | (PCC) | (SCC) |  | (6.02N+1.76) |
|  |  |  |  |  | Tx leakage mapping to |
|  |  |  |  |  | -12dBFS (PAPR is considered) |
| PA out | 27 | / | / | / | / |
| Duplexer isolation | NA | / | / | / | / |
| Rx Antenna input | / | -94.8 | -94.8 | / | / |
| FE loss | / | 5 | 5 | / | / |
| Duplexer output | NA | -99.8 | -99.8 | / | / |
| LPF attenuation | 0 | 1 | 1 | / | / |
| LPF output (dBm) | NA | -100.8 | -100.8 | / | / |
| Noise floor for basic REFSENS | / | / | / | -99.8 | / |
| NF increase due to AGC adjustment | / | / | / | 0 | / |
| Noise floor | / | / | / | -99.8 | / |
| In-band noise caused by Tx leakage (Derived based on MSD and -1dB SNR) | / | / | -99.8 | / | / |
| ADC output (dBm) | / | / | / | / | NA |
| SNR after ADC (dB) | / | / | / | / | 0(PCC) |
|  |  |  |  |  | 0(SCC) |

7.1.6.2	Case 2, 40 + 40 MHz

Table 7.1.6-2 Link budget of Self-interference for 40+40 MHz on band n41

|  | Tx leakage | Wanted Carrier | Wanted Carrier | Noise floor | ADC noise (11 bit): -68.0dBFs |
| --- | --- | --- | --- | --- | --- |
|  |  | (PCC) | (SCC) |  | (6.02N+1.76) |
|  |  |  |  |  | Tx leakage mapping to |
|  |  |  |  |  | -12dBFS (PAPR is considered) |
| PA out | 27 | / | / | / | / |
| Duplexer isolation | NA | / | / | / | / |
| Rx Antenna input | / | -88.6 | -88.6 | / | / |
| FE loss | / | 5 | 5 | / | / |
| Duplexer output | NA | -93.6 | -93.6 | / | / |
| LPF attenuation | 0 | 1 | 1 | / | / |
| LPF output (dBm) | NA | -94.6 | -94.6 | / | / |
| Noise floor for basic REFSENS | / | / | / | -93.6 | / |
| NF increase due to AGC adjustment | / | / | / | 0 | / |
| Noise floor | / | / | / | -93.6 | / |
| In-band noise caused by Tx leakage (Derived based on MSD and -1dB SNR) | / | / | -93.6 | / | / |
| ADC output (dBm) | / | / | / | / | NA |
| SNR after ADC (dB) | / | / | / | / | 0(PCC) |
|  |  |  |  |  | 0(SCC) |

7.1.7	Summary results for example bands

Evaluated results are summarized in the table below:

Table 7.1.7-1: Proposed ΔRIBNC for example bands on Rx RF chain mode

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | PCC ΔRIBNC (dB) | SCC ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n2(2A) | 15/15 | 5MHz + 5MHz | Wgap = 50.0 | 10 | 12 | 12.8 | FDD |
|  |  |  | Wgap = [10] | 25 | 0 | 0 |  |
| CA_n3(2A) | 15/15 | 5MHz + 5MHz | Wgap = 65.0 | 12 | 12.2 | 12.9 | FDD |
|  |  |  | Wgap = [20] | 25 | 0 | 0 |  |
| CA_n7(2A) | 15/15 | 10MHz + 5MHz | Wgap = 55.0 | 32 | 6.4 | 6.4 | FDD |
|  |  |  | Wgap = [25] | 25 | 0 | 0 |  |
| CA_n25(2A) | 15/15 | 5MHz + 5MHz | Wgap = 55.0 | 10 | 13.8 | 14.3 | FDD |
|  |  |  | Wgap = [10] | 25 | 0 | 0 |  |
| CA_n25(2A) | 15/15 | 40MHz + 5MHz | Wgap = 20.0 | 40 (RBstart = 176) | 6.5 | 24.9 | FDD |
| CA_n26(2A) | 15/15 | 15MHz + 10MHz | Wgap = 10.0 | 5 (RBstart = 74) | 13.9 | 26.2 | FDD |
| CA_n41(2A) | 15/15 | 10MHz + 10MHz | Wgap = 80.0 | N/A | 0 | 0 | TDD |
| CA_n41(2A) | 15/15 | 40MHz + 40MHz | Wgap = 20.0 | N/A | 0 | 0 | TDD |

7.1.8 Discussion on in-gap requirements

In last RAN4 meeting, it was agreed evaluate performance impact on the cases:

a	image of in-gap interference overlaps with wanted carrier

-	For the cases, image of ACS/IBB1/IBB2/NBB overlaps with wanted CC

b	image of in-gap blocking interference does not overlap with wanted carrier

Let’s take a look on the potential performance impact with the agreed assumptions. It can be seen for example, a DL NCCA composed of unequal CBW, such as 40+5MHz. When testing in-gap ACS/IBB1/IBB2 on CC0, if BW of CC0 is larger than CC1, the image of in-gap interference may also fall into co-channel of CC0 that result in performance degradation as illustrated in Figure 7.1.8-1. The agreed assumption of Rx chain image rejection is 25dB and general assumption of receiver SNR is -1dB.

For the cases image of in-gap ACS/IBB1/IBB2/NBB overlaps with wanted CC, the maximum blocker to wanted signal power ratio shall not be larger than 26dB.

Figure 7.1.8-1: Illustration on impacts of the unwanted/interference image fall into co-channel CC

Then we also calculate the ACS case1 blocker level and the results are summarized in the table below:

Table 7.1.8-1: ACS case 1 blocker level for example bands on Rx RF chain mode

| CA configuration | SCS | UL PCC allocation | PCC REFSENS | PCC | ACS case1 PCC | SCC REFSENS | SCC | ACS case1 SCC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (PCC/SCC) | (LCRB) | 15KHz SCS | ΔRIBNC (dB) |  | 15KHz SCS | ΔRIBNC (dB) |  |
|  | (kHz) |  |  |  |  |  |  |  |
| CA_n2(2A) | 15/15 | 10 | -98 | 12 | -40.5 | -98 | 12.8 | -39.7 |
| CA_n3(2A) | 15/15 | 12 | -97 | 12.2 | -39.3 | -97 | 12.9 | -38.6 |
| CA_n7(2A) | 15/15 | 32 | -94.8 | 6.4 | -42.9 | -98 | 6.4 | -46.1 |
| CA_n25(2A) | 15/15 | 10 | -96.5 | 13.8 | -37.2 | -96.5 | 14.3 | -36.7 |
| CA_n25(2A) | 15/15 | 40 (RBstart = 176) | -79.5 | 6.5 | -36.5 | -96.5 | 24.9 | -26.1 |
| CA_n26(2A) | 15/15 | 5 (RBstart = 74) | -92.7 | 13.9 | -36.3 | -94.5 | 26.2 | -22.8 |
| CA_n41(2A) | 15/15 | N/A | -94.8 | 0 | -49.3 | -94.8 | 0 | -49.3 |
| CA_n41(2A) | 15/15 | N/A | -88.6 | 0 | -52.1 | -88.6 | 0 | -52.1 |

From the calculation we can see the ACS case 1 blocker level are mostly less than that of Tx leakage except baand n26. In the current TS38.101-1, it has been specified “The UE shall fulfil the minimum requirements all values of a single adjacent channel interferer in-gap and out-of-gap up to a –25 dBm interferer power while all downlink carriers are active”. The UE design shall at least fulfil above requirement so it would be able to pass the -25dBm maximum acceptable absolute power of in-gap and out-of-gap interferers.

For the cases image of ACS/IBB/IBB2/NBB does not overlap with wanted CC, no relaxation would be needed based on evaluated ΔRIBNC on both PCC and SCC. Referring to current specification, the maximum acceptable absolute power of in-gap interferer should be less than or equal to -25dBm

On the other hand, when measuring ACS/IBB1/IBB2 of CC0, the image of interference may fall into the co-channel range of CC1. Though this may not be an issue for the conformance test since the conformance test does measure REFSENS of CC1 when testing ACS/IBB1/IBB2 of CC0, this may be an implementation issue in the field as illustrated in Figure 7.1.8-2.

Figure 7.1.8-2: Illustration on impacts of the unwanted/interference image fall into the other CC

7.2	RF requirements evaluation from company B

With common RF chain architecture, the TX noise at SCC from the PCC remains the same as the analog BB filter cannot provide any help. However, besides TX noise, there is TX leakage at TX band that may benefit from analog filtering before it goes into digital BB. In this case, since common RF chain architecture uses a wide analog BB filter to cover the overall frequency span from the lower edge of CC1 to the upper edge of CC2, as compared to the separate RF chain architecture where two separate narrow BB filters to cover CC1 and CC2, there is less filter rejection. As a result, both the PCC and SCC performance may take some hit[19].

We first provided analysis on the impact of self-interference, i.e., the NF increase due to AGC adjustment (i.e., RX gain backoff). Table 1 and Table 2 show our analysis for CA configuration CA_n3(2A) (5MHz PCC+ 5MHz SCC, Wgap = 65MHz) and CA_n25(2A) (5MHz PCC + 5MHz SCC, Wgap = 55MHz), respectively.

Table 7.2-1: Link budget of Self-interference for CA_n3(2A)

|  | Tx leakage | Wanted Carrier (PCC) | Wanted Carrier (SCC) | Noise floor | ADC noise included in noise floor |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Tx leakage mapping to a suitable setpoint (PAPR is considered) |
| PA out | 27 | - | - | - | - |
| Duplexer isolation | 50 | - | - | - | - |
| Rx Antenna input | - | -97 | -92.3 | - | - |
| FE loss | - | 4 | 4 | - | - |
| Duplexer output | -23 | -101 | -96.3 | - | - |
| LPF attenuation | 13.4 | 1 | 1 | - | - |
| LPF output (dBm) | -36.4 | -102 | -97.3 | - | - |
| Noise floor for basic REFSENS | - | - | - | -101 | - |
| NF increase due to AGC adjustment (including effects due to ADC setpoint adjustment) | - | - | - | 10.5 | - |
| Noise floor (including ADC noise) | - | - | - | -90.5 | - |
| In-band noise caused by Tx (Derived based on MSD and SNR = -1dB) | - | - | -98.1 | - | - |
| SNR after ADC (dB) | - | - | - | - | -11.5(PCC) |
|  |  |  |  |  | -7.5(SCC) |

Table 7.2-2: Link budget of Self-interference for CA_n25(2A)

|  | Tx leakage | Wanted Carrier (PCC) | Wanted Carrier (SCC) | Noise floor | ADC noise included in noise floor |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Tx leakage mapping to a suitable setpoint (PAPR is considered) |
| PA out | 27 | - | - | - | - |
| Duplexer isolation | 50 | - | - | - | - |
| Rx Antenna input | - | -96.5 | -91.5 | - | - |
| FE loss | - | 4 | 4 | - | - |
| Duplexer output | -23 | -100.5 | -95.5 | - | - |
| LPF attenuation | 12.9 | 1 | 1 | - | - |
| LPF output (dBm) | -35.9 | -101.5 | -96.5 | - | - |
| Noise floor for basic REFSENS | - | - | - | -100.5 | - |
| NF increase due to AGC adjustment (including effects due to ADC setpoint adjustment) | - | - | - | 11 | - |
| Noise floor (including ADC noise) | - | - | - | -89.5 | - |
| In-band noise caused by Tx (Derived based on MSD and SNR = -1dB) | - | - | -98.1 | - | - |
| SNR after ADC (dB) | - | - | - | - | -12(PCC) |
|  |  |  |  |  | -7.7(SCC) |

Next, we consider the impact of blocker in the gap in addition to the self-interference. In the analysis, we consider two cases, where the image of the blocker overlaps and does not overlap with a wanted CC.

Case 1: The image of the blocker does not overlap with a wanted CC

Based on our analysis, the results are shown in Table 3 and Table 4 for CA_n3(2A) and CA_n25(2A), respectively.

Note the following in the analysis:

1	In ACS Case 1, both the receiver sensitivity (REFSENS) and blocker power are increased by the proposed ΔRIBNC adjustment due to self-interference.

2	adjustment due to self-interference, but the blocker power is not. This is because in ACS Case 2, the wanted signal power is fixed at -56.5dBm and the blocker power is fixed at -25dBm in ACS Case 2, -56dBm in IBB Case 1, and -44dBm in IBB Case 2, respectively.

Table 7.2-3: Link budget of Blocker for CA_n3(2A)

| Duplexer isolation (dB) | 50 |  |
| --- | --- | --- |
| LPF attenuation (dB) | 13.4 |  |
| NF increase due to AGC adjustment (dB) | 10.5 |  |
| Proposed ΔRIBNC adjustment due to self-interference | 10.5dB for PCC | 6.5dB for SCC |
|  | PCC SNR (dB) after ADC | SCC SNR (dB) after ADC |
| REFSENS | -1 | -1 |
| ACS Case 1 | 13 | 13 |
| ACS Case 2 | 4.1 | 4.1 |
| IBB Case 1 | 5 | 5 |
| IBB Case 2 | 5 | 5 |

Table 7.2-4: Link budget of Blocker for CA_n25(2A)

| Duplexer isolation (dB) | 50 |  |
| --- | --- | --- |
| LPF attenuation (dB) | 12.9 |  |
| NF increase due to AGC adjustment (dB) | 11 |  |
| Proposed ΔRIBNC adjustment due to self-interference | 11dB for PCC | 6.7dB for SCC |
|  | PCC SNR (dB) after ADC | SCC SNR (dB) after ADC |
| REFSENS | -1 | -1 |
| ACS Case 1 | 13 | 13 |
| ACS Case 2 | 4.1 | 4.1 |
| IBB Case 1 | 5 | 5 |
| IBB Case 2 | 5 | 5 |

Based on the analysis, with the proposed ΔRIBNC adjustment due to self-interference, the ACS/IBB requirements can be met with some margin. However, we note that this conclusion may depend on the CA band combination. For instance, if the duplex gap between UL and DL bands are wide enough, in other words, the TX leakage is of little issue and little ΔRIBNC adjustment is needed to meet the REFSENS requirement, there may be some adjustment needed for ACS/IBB requirements.

Case 2: The image of the blocker overlaps with a wanted CC

When the image of the block overlaps with a wanted CC, the results are shown in Table 5 and Table 6 for CA_n3(2A) and CA_n25(2A), respectively.

Note the following for the analysis:

1	Image rejection ratio at the UE is assumed to be 25dB.

2	Other assumptions are the same as in Case 1 where the image of the blocker does not overlap with a wanted CC.

For the two CA configurations, CA_n3(2A) (5MHz PCC + 5MHz SCC, Wgap = 65MHz) and CA_n25(2A) 3	(5MHz PCC + 5MHz SCC, Wgap = 55MHz), because the PCC and SCC have the same channel bandwidth, the image of the blocker does not overlap with a wanted CC. Therefore, the analysis shown below are aimed to provide an initial assessment of the impact. Further analysis is needed to address the CA configurations where the image of the blocker does overlap with a wanted CC.

Table 7.2-5: Link budget of Blocker for CA_n3(2A)

| Image rejection ratio (IRR) (dB) | 25 |  |
| --- | --- | --- |
|  | PCC SNR (dB) after ADC | SCC SNR (dB) after ADC |
| REFSENS | -1 | -1 |
| ACS Case 1 | -6.5 | -6.5 |
| ACS Case 2 | -6.5 | -6.5 |
| IBB Case 1 | 0.5 | 1.2 |
| IBB Case 2 | -11.5 | -10.8 |

Table 7.2-6: Link budget of Blocker for CA_n25(2A)

| Image rejection ratio (IRR) (dB) | 25 |  |
| --- | --- | --- |
|  | PCC SNR (dB) after ADC | SCC SNR (dB) after ADC |
| REFSENS | -1 | -1 |
| ACS Case 1 | -6.5 | -6.5 |
| ACS Case 2 | -6.5 | -6.5 |
| IBB Case 1 | 1.5 | 2.2 |
| IBB Case 2 | -10.5 | -9.8 |

It can be seen that the SNR for ACS Case 1, ACS Case 2 and IBB Case 2 is 5-10dB below -1dB, showing the image of the blocker causes strong interference to the receiver.

Another way to address the image interference issue is to limit the blocker to wanted CC power ratio of 26dB. If this is the case, SNRs for ACS/IBB Case 1 and 2 in Table 5 and Table 6 will be -1dB or higher.

7.x Summary of evaluation results

< to be updated>

8	Study of means for a UE to inform the network of appropriate CA configuration it can support

It has been agreed to consider the following aspects for how to indicate UE capability for supporting ”one Rx RF chain” for DL fragmented carriers[22]:

  - Alternative 1: use the legacy band combination reporting scheme + Rx sharing capability indication

    - Alternative 1 has no modification of CA configurations in RAN4

  - Alternative 1a: use the legacy band combination reporting scheme + Rx sharing capability and other related information including the frequency separation between two carriers.

    - Alternative 1a has no modification of CA configurations in RAN4

  - Alternative 2: use the new notation for CA configurations with single Rx RF Chain

  - Alternative 2a: use the new bandwidth class conditioned on the frequency separation between two carriers

RAN4 also agreed to clarify CA fallback and architecture switching as following aspects:

  - CA fallback with/without UE architecture switching

    - Consider the fall back

      - from high order CA to lower order CA

      - from CA to single carrier mode

    - Decided by network

  - UE architecture switching:

    - Switching between 2 Rx RF chains and 1 Rx RF chain on CCs of intra-band NC CA

When discussing UE behaviours, it is also agreed to consider triggering condition to enable the “one Rx RF chain” mode as following aspects:

- Option 1: Rely on existing channel quality reporting and procedures. No new indication from UE is needed

- Option 2: The UE need the measurement to determine and indicate to the network whether it can support non-contiguous CCs with one RX RF chain or not, assuming the associated degradation is no greater than allowed degradation that is specified

Note: detailed conditions to enable/disable can be further discussed and decided separately

8.1	Indication of one Rx RF chain mode for DL fragmented CA

[8.1.1	Indication of fragmented CA alternative 1: Legacy band combination reporting scheme + Rx sharing capability indication]

< to be updated>

[8.1.2	Indication of fragmented CA alternative 1a: Legacy band combination reporting scheme + Rx sharing capability and other related information]

< to be updated>

[8.1.3	Indication of fragmented CA alternative 2: New notation for CA configurations with single Rx RF Chain]

< to be updated>

8.1.4	Indication of fragmented CA alternative 2a: use the new bandwidth class conditioned on the frequency separation between two carriers

To allow both the UE and the network separating fragmented carrier operation from other types of CA configurations a different indication is suggested during the study phase.

A proposal is to use the currently undefined NR CA bandwidth class (BCS) ‘F’ should be introduced for indicating a CA configuration with fragmented carrier operation.

TS 38.101-1: Table 5.3A.5-1: NR CA bandwidth classes

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel ≤ BWChannel,max | 1 | 1, 2, 34, 4 |
| B | 20 MHz ≤ BWChannel_CA ≤ 100 MHz | 2 | 2, 34 |
| C | 100 MHz < BWChannel_CA ≤ 2 x BWChannel,max | 2 | 1, 34 |  |
| D | 200 MHz < BWChannel_CA ≤ 3 x BWChannel,max | 3 |  |  |
| E | 300 MHz < BWChannel_CA ≤ 4 x BWChannel,max | 4 |  |  |
| F | BWChannel ≤ 100 MHz | 1, NOTE 5 | 4 |  |
| G | 100 MHz < BWChannel_CA ≤ 150 MHz | 3 | 2 |  |
| H | 150 MHz < BWChannel_CA ≤ 200 MHz | 4 |  |  |
| I | 200 MHz < BWChannel_CA ≤ 250 MHz | 5 |  |  |
| J | 250 MHz < BWChannel_CA ≤ 300 MHz | 6 |  |  |
| K | 300 MHz < BWChannel_CA ≤ 350 MHz | 7 |  |  |
| L | 350 MHz < BWChannel_CA ≤ 400 MHz | 8 |  |  |
| M3 | 50 MHz ≤ BWChannel_CA ≤ 200 MHz | 3 | 34 |  |
| N3 | 80 MHz ≤ BWChannel_CA ≤ 300 MHz | 4 |  |  |
| O3 | 100 MHz ≤ BWChannel_CA ≤ 400 MHz | 5 |  |  |
| NOTE 1: BWChannel, max is maximum channel bandwidth supported among all bands in a release NOTE 2: It is mandatory for a UE to be able to fallback to lower order NR CA bandwidth class configuration within a fallback group. It is not mandatory for a UE to be able to fallback to lower order NR CA bandwidth class configuration that belong to a different fallback group. NOTE 3: This bandwidth class is only applicable to bands identified for use with shared spectrum channel access in Table 5.2-1. NOTE 4: Fallback group 3 is only applicable to bands identified for use with shared spectrum channel access in Table 5.2-1. NOTE 5: For fragmented carrier operation with a single contiguous frequency range configured for the UE with multiple fragmented carriers as indicated by IE [FragmentedCarrier]. |  |  |  |

In the proposal above, where the newly added parts are highlighted with yellow, F is inserted with a corresponding number of contiguous CCs as ‘1’, as the intention is to treat the multiple fragmented carriers as a single CC regarding the number of occupied Rx RF chains. Adding Note 5 clarifies that this single contiguous CC can consist of multiple fragmented carriers assumed occupying 1 Rx chain. The new BCS, F, is then added with fallback group 4, which is added correspondingly for the single CC BCS, A, as the fallback from fragmented carrier operation would be single CC operation. However, if the UE can fallback to 2 Rx RF chains from single Rx RF chain (due to high in-gap interference) this should also be possible.

By adding the new NR CA bandwidth class, this can be used to identify fragmented carrier operation and the number of fragments. Examples using band n25 with two fragments are shown in Fig 1.

Fig.  8.1-1: Example of CA_n25(2F)

8.2	CA fallback and architecture switching

< to be updated>

8.3	Triggering condition to enable ”One Rx RF chain”

8.3.1 Triggering condition proposal for option 1: Rely on existing channel quality reporting and procedures

< to be updated>

8.3.2 Triggering condition proposal for option 2: UE measurement to determine and indicate to the network

< to be updated>

9	Summary for NR FR1 DL fragment carriers study

< to be updated>

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | Tdoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-08 | RAN4 #112 | R4-2411553 |  |  |  | TR skeleton for Study on NR FR1 DL Fragmented Carriers | 0.0.0 |
| 2024-08 | RAN4 #112bis | R4-2416132 |  |  |  | Editorial update of TR for Study on NR FR1 DL Fragmented Carriers | 0.0.1 |
| 2025-03 | RAN4 #114bis | R4-2504105 |  |  |  | Updated version including agreed TP: R4-2502925 TP for TR38.755 | 0.1.0 |
| 2025-03 | RAN4 #114bis | R4-2505158 |  |  |  | Updated version including agreed TPs | 0.2.0 |
| 2025-5 | RAN4#115 | R4-25xxxxx |  |  |  | Reorganize directory based on WF in RAN4#114bis | 0.3.0 |