| 3GPP TR 38.793 V0.2.0 (2025-09) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Requirements for simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	6

4	Background	7

4.1	Background information in Rel-17 and Rel-18	7

4.2	WI Objective in Rel-19	7

4.3	Applicability of mandatory support of simultaneous Rx-Tx	8

5	Specific Band combinations	8

5.1	CA_n40A-41A	8

5.1.1	Status of the band combination	8

5.1.1.1	Operating bands for CA	8

5.1.1.2	Power class of the BC	8

5.1.1.3	∆TIB and ∆RIB	9

5.1.2	MSD analysis for simultaneous Rx/Tx	9

5.1.3	Requirements for simultaneous Rx/Tx	10

Annex A (informative): Change history	11

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document is a technical report for simultaneous Rx/Tx band combinations in Rel-18. The TR includes ...

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: “Vocabulary for 3GPP Specifications”.

[2]	RP-222648, “New WID: Simultaneous Rx/Tx inter-band combinations for NR CA/DC, NR SUL and LTE/NR DC in Rel-18”.

[3]	3GPP TS 38.101-1: “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

[4]	3GPP TS 38.101-2: “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”.

[5]	3GPP TS 38.101-3: “NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”..

…

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

# 4	Background

## 4.1	Background information in Rel-17 and Rel-18

Simultaneous Rx/Tx capability for inter-band CA, SUL and EN-DC band combinations were introduced from Rel-15. Specifically, for inter-band CA and EN-DC combination, the capability is used for TDD-TDD and TDD-FDD band combinations. According to the description of the capability, it is conditional mandatory and the condition is described in the field, i.e. indicated in the RAN4 spec which combinations should mandatorily support simultaneous Rx/Tx. For the combinations which have no such indication, the capability is optional, i.e. for UE supporting simultaneous Rx/Tx, the capability should be reported, otherwise, the capability is absent or not reported. Since the capability is important for network scheduling, it should be reported accurately.

In Rel-17, the principles for judging the mandatory capability for a band combination have been discussed, and the cases include:

-	FR1+FR1 FDD-TDD band combination

-	FR1+FR1 TDD-TDD band combination

-	FR1+FR2 FDD-TDD band combination

-	FR1+FR2 TDD-TDD band combination

-	FR2+FR2 TDD-TDD band combination

For some categories, the capability should be checked case by case. Therefore, to facilitate the analysis of specific band combinations, a dedicated WI is needed to avoid the ambiguity for application of the general principles agreed in Rel-17 and the analysis and conclusion should be captured in the TR.

In addition, as the capability is defined for CA, SUL, MR-DC and NR-DC band combinations, and applicability of the corresponding requirements cover different specifications, e.g. TS 38.101-1 and TS 38.101-3, the way to treat simultaneous Rx/Tx capability as well as the requirements should be aligned among the specifications.

The fallback rules are captured as below for reference:

-	Request for additions of band combinations to this WI shall be provided using an agreed template and sent to the 3GPP_TSG_RAN_WG4_NR_BANDS email reflector before a RAN4 Tdoc submission deadline and no new band combinations are allowed to be requested after the deadline except to correct the missing fallback and add more supporting companies for the proposed band combinations.

-	When a proponent requests a new band combination, all the next level fallback configurations shall be listed and recorded in the request template and the status (“New”, “Ongoing”, “Completed”) of all the fallback configurations shall be declared accurately and clearly. For “New” fallback configurations, the proponent shall ensure these fallback configurations are also requested together with the higher order band combination in the same meeting.

-	A band combination configuration can only be considered as completed when all of the fallback configurations are completed and specified in advance or at the same meeting. It is the responsibility of the proponent to ensure the status of all of the fallback mode configurations. Rapporteurs and other companies are encouraged to check the status of all of the fallback configurations once the higher order band combinations are declared as completed.

## 4.2	WI Objective in Rel-19

1.	Identify the unnecessary Notes for simultaneous Rx-Tx applied to band combinations for NR CA/DC, NR SUL and LTE/NR DC. Focus on principles and specific methods of simplification on Notes to simultaneous Rx-Tx case by case.

2.	Clarify in the specification on the simultaneous Rx-Tx feature for DC band combinations with same band but different configurations of carrier number.

3.	Identify feasibility for each requested FDD-TDD and TDD-TDD band combinations for CA, SUL, MR-DC supporting simultaneous Rx/Tx capability/operation based on technical analysis, especially for those with large MSD values. If needed, specify the MSD requirements for the identified band combinations.

Note 1: Band combinations considered in this WI have to be introduced first via basket WIs (see 2.3) or completed in previous releases if necessary.

Note 2: Whether the simultaneous Rx-Tx capability could be supported or not depends on the evaluation of MSD for the requested band combinations case by case.

Note 3: Applicability of simultaneous Rx/Tx capability to FDD-TDD band pairs should be reviewed

4.	Align the specification treatment of simultaneous Rx/Tx capability for CA, SUL, MR-DC and NR-DC band combinations.

## 4.3	Applicability of mandatory support of simultaneous Rx-Tx

During Rel-19 RAN4 has agreed the following applicability of mandatory support of simultaneous Rx-Tx which may aid as a guide to when requirements for simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC as captured in this TR is needed defined.

RAN4 has concluded simultaneous Rx/Tx requirements for different types of inter-band CA and SUL configurations:

- TDD + TDD -> Conditional mandatory support for simultaneous RxTx with signaling

- FDD + TDD -> Mandatory support for simultaneous RxTx with signaling

- TDD + SDL -> Mandatory support for simultaneous RxTx with signaling

- FDD/TDD + SUL -> Mandatory support for simultaneous RxTx with signaling

- FDD + FDD -> Mandatory support for simultaneous RxTx without signaling

- FDD + SDL -> Mandatory support for simultaneous RxTx without signaling

Conditional mandatory support means that explicit indication per band combinations such as e.g. in TS 38.101-1 Table 5.2A.2.1 by Note 1 is needed to mandate the support for simultaneous RxTx with signaling.

In case overlapping FDD band configurations are introduced, such cases will be handled in a case-by-case manner.

# 5	Specific Band combinations

## 5.1	CA_n40A-41A

### 5.1.1	Status of the band combination

#### 5.1.1.1	Operating bands for CA

The operating bands for CA_n40-n41 are specified in Table 5.3.1.1-1.

Table 5.3.1.1-1: Inter-band CA operating bands involving FR1

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 8) |
| --- | --- | --- |
| CA_n40-n41 | n40, n41 | No |
| NOTE 8: Applicable when dynamic Tx switching is conducted. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13]. |  |  |

For CA_n40-n41, the current spec only considers scenarios without simultaneous Rx/Tx, which are limited by the applicable requirements of ΔTIB,c and ΔRIB,c.

#### 5.1.1.2	Power class of the BC

The UE Power Class for uplink CA_n40A-n41A are specified in Table 5.3.1.2-1. The supported power class of the band combination would have impact on the MSD, if any, based on the following study.

Table 5.3.1.2-1 UE Power Class for uplink CA_n40A-n41A

| Uplink CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n40A-n41A |  |  | 266,7 | +2/-3 | 23 | +2/-3 |  |  |
| NOTE 9: The UE supports either PC3 or PC2 within each NR TDD band. |  |  |  |  |  |  |  |  |

#### 5.1.1.3	∆TIB and ∆RIB

The ΔTIB,c due to CA_n40-n41 are specified in Table 5.3.1.3-1.

Table 5.3.1.3-1: ΔTIB,c due to CA_n40-n41

| Inter-band CA combination | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n40-n41 | n40 | 0.5 |
|  | n41 | 0.5 |

ΔRIB,c due to CA_n40-n41 is set to zero.

It is noted that ΔTIB,c and ΔRIB,c reflect the possible UE architecture to support the CA band combination.

### 5.1.2	MSD analysis for simultaneous Rx/Tx

Table 1-1 PC3 MSD for simultaneous Rx-Tx CA_n40A-n41A [1][2]

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2345 | 50 | 30 | 128 (RBstart=5) | 2565 | 100 | [11.8] | >ACLR2 |
| n41 | n40 | 2565 | 100 | 30 | 270 (RBstart=0) | 2345 | 50 | [28.6] | ACLR2 |

Table 1-2 PC2 MSD for simultaneous Rx-Tx CA_n40A-n41A

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2345 | 50 | 30 | 128 (RBstart=5) | 2565 | 100 | [14.4] | >ACLR2 |
| n41 | n40 | 2565 | 100 | 30 | 270 (RBstart=0) | 2345 | 50 | [31.6] | ACLR2 |

Table 2-1 PC3 MSD for simultaneous Rx-Tx CA_n40A-n41A test point [Apple R4-2500305]

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2345 | 50 | 30 | 128 (RBstart=5) | 2565 | 100 | 12.7 | >ACLR2 |
| n41 | n40 | 2565 | 100 | 30 | 270 (RBstart=0) | 2345 | 50 | 28.9 | ACLR2 |

Table 2-2 PC2 MSD for simultaneous Rx-Tx CA_n40A-n41A test point

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2345 | 50 | 30 | 128 (RBstart=5) | 2565 | 100 | 15.5 | >ACLR2 |
| n41 | n40 | 2565 | 100 | 30 | 270 (RBstart=0) | 2345 | 50 | 32.0 | ACLR2 |

Table 3-1 PC3 MSD for simultaneous Rx-Tx CA_n40A-n41A [Qualcomm R4-2501576]

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2345 | 50 | 30 | 128 (RBstart=5) | 2565 | 100 | 12 | >ACLR2 |
| n41 | n40 | 2565 | 100 | 30 | 270 (RBstart=0) | 2345 | 50 | 26.8 | ACLR2 |

Table 3-2 PC2 MSD for simultaneous Rx-Tx CA_n40A-n41A

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2345 | 50 | 30 | 128 (RBstart=5) | 2565 | 100 | 14.7 | >ACLR2 |
| n41 | n40 | 2565 | 100 | 30 | 270 (RBstart=0) | 2345 | 50 | 29.8 | ACLR2 |

Table 4-1 PC3 MSD for simultaneous Rx\Tx CA_n40A-n41A [Skyworks R4-2501953]

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41x | 2345 | 50 | 30 | 128 (RBstart=5) | 2565 | 100 | [5.7] | >ACLR2 |
| n41 | n40x | 2565 | 100 | 30 | 270 (RBstart=0) | 2345 | 50 | [18.4] | ACLR2 |
| Note X: The exceptional UL/DL configurations account for regional spectrum allocation considerations. |  |  |  |  |  |  |  |  |  |

Table 4-2 PC2 1Tx MSD for simultaneous Rx\Tx CA_n40A-n41A

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41x | 2345 | 50 | 30 | 128 (RBstart=5) | 2565 | 100 | [7.7] | >ACLR2 |
| n41 | n40x | 2565 | 100 | 30 | 270 (RBstart=0) | 2345 | 50 | [21.4] | ACLR2 |
| Note X: The exceptional UL/DL configurations account for regional spectrum allocation considerations. |  |  |  |  |  |  |  |  |  |

### 5.1.3	Requirements for simultaneous Rx/Tx

Table 3-1-9 PC3 MSD for simultaneous Rx-Tx CA_n40A-n41A

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2345 | 50 | 30 | 128 (RBstart=5) | 2565 | 100 | 11.2 | >ACLR2 |
| n41 | n40 | 2565 | 100 | 30 | 270 (RBstart=0) | 2345 | 50 | 27.1 | ACLR2 |

Table 3-1-10 PC2 MSD for simultaneous Rx-Tx CA_n40A-n41A

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2345 | 50 | 30 | 128 (RBstart=5) | 2565 | 100 | 13.9 | >ACLR2 |
| n41 | n40 | 2565 | 100 | 30 | 270 (RBstart=0) | 2345 | 50 | 30.1 | ACLR2 |

Note 1: The exceptional UL/DL configurations account for regional spectrum allocation considerations.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-02 | RAN4#114 | R4-2500148 |  |  |  | TR skeleton | 0.0.1 |
| 2025-02 | RAN4#114 | R4-2503018 |  |  |  | TP for TR 38.793 CA_n40A-n41A | 0.0.1 |
| 2025-04 | RAN4#114bis | R4-2503143 |  |  |  | TR v0.1.0 | 0.1.0 |
| 2025-08 | RAN4#116 | R4-2511899 |  |  |  | TP to TR 38.793 on applicability of mandatory support of simultaneous Rx-Tx | 0.2.0 |
| 2025-09 | RAN4#116 | R4-2510471 |  |  |  | TR v0.2.0 | 0.2.0 |