| 3GPP TR 38.830 V0.0.2 (2020-08) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on NR coverage enhancements (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	6

4	Evaluation methodology	7

5	Baseline coverage performance	7

5.1	Baseline coverage performance for FR1	7

5.2	Baseline coverage performance for FR2	7

6	Potential techniques for coverage enhancements	8

[6.1	PUSCH coverage enhancements]	8

[6.2	PUCCH coverage enhancements]	8

[6.3	Coverage enhancements for channels other than PUSCH and PUCCH]	8

7	Conclusions	8

Annex <A>: Simulation assumptions	9

A.1	Simulation assumptions for FR1	9

A.2	Simulation assumptions for FR2	13

A.3	Link budget template	17

Annex <B> (informative): Change history	18

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

In RAN #86 meeting, a new Rel-17 study item on NR coverage enhancements was approved [2]. The objective of this study item is to study potential coverage enhancement solutions for specific scenarios for both FR1 and FR2. The detailed objectives are as follows.

-	The target scenarios and services include

-	Urban (outdoor gNB serving indoor UEs) scenario, and rural scenario (including extreme long distance rural scenario) for FR1

-	Indoor scenario (indoor gNB serving indoor UEs), and urban/suburban scenario (including outdoor gNB serving outdoor UEs and outdoor gNB serving indoor UEs) for FR2.

-	TDD and FDD for FR1.

-	VoIP and eMBB service for FR1.

-	eMBB service as first priority and VoIP as second priority for FR2.

-	LPWA services and scenarios are not included.

-	Identify baseline coverage performance for both DL and UL for the above scenarios and services based on link-level simulation

-	UL channels (including PUSCH and PUCCH) are prioritized for FR1.

-	Both DL and UL channels for FR2.

-	Identify the performance target for coverage enhancement, and study the potential solutions for coverage enhancements for the above scenarios and services

-	The target channels include at least PUSCH/PUCCH

-	Study enhanced solutions, e.g., time domain/frequency domain/DM-RS enhancement (including DM-RS-less transmissions)

-	Study the additional enhanced solutions for FR2 if any

-	Evaluate the performance of the potential solutions based on link level simulation.

# 1	Scope

The present document captures the results and findings from the study item "New SID on NR coverage enhancement" [2]. The purpose of this TR is to document the baseline coverage performance for both FR1 and FR2 considering the scenarios and services identified in [2], and to document the evaluation and findings of the potential enhancements for the identified scenarios and services.

This activity involves the Radio Access work area of the 3GPP studies and has potential impacts both on the Mobile Equipment and Access Network of the 3GPP systems.

This document is a 'living' document, i.e. it is permanently updated and presented to TSG-RAN meetings.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP RP-193240: "New SID on NR coverage enhancement".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ACK	Acknowledgement

BWP	Bandwidth Part

BS	Base Station

CSI	Channel State Information

CCE	Control Channel Element

DL	Downlink

DMRS	Dedicated Demodulation Reference Signals

eMBB	enhanced Mobile BroadBand

FDD	Frequency Division Duplex

gNB	NR Node B

HARQ	Hybrid Automatic Repeat reQuest

iBLER	initial BLock Error Rate

MCS	Modulation and Coding Scheme

NACK	Negative Acknowledgement

OS	OFDM symbol

PDCCH	Physical Downlink Control Channel

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

PDSCH	Physical Downlink Shared Channel

PRACH	Physical Random Access Channel

PRB	Physical Resource Block

rBLER	residual BLock Error Rate

SCS	Subcarrier Spacing

SR	Scheduling Request

SSB	Synchronization Signal Block

TBS	Transport Block Size

TDD	Time Division Duplex

UCI	Uplink Control Information

UE	User Equipment

UL	Uplink

# 4	Evaluation methodology

For this study item, the basic evaluation methodology is based on link-level simulation.

-	Step 1: Obtain the required SINR for the physical channels under target scenarios and service/reliability requirements.

The simulation assumptions for step 1 are provided in Annex A.1 for FR1 and A.2 for FR 2 respectively.

-	Step 2: Obtain the baseline performance based on required SINR and link budget template.

The parameter assumptions of the link budget template for step 2 are provided in Annex A.3.

The evaluation methodology based on system-level simulation is optional and the simulation assumptions for system-level simulation are up to companies' reports.

# 5	Baseline coverage performance

## 5.1	Baseline coverage performance for FR1

## 5.2	Baseline coverage performance for FR2

# 6	Potential techniques for coverage enhancements

## [6.1	PUSCH coverage enhancements]

## [6.2	PUCCH coverage enhancements]

## [6.3	Coverage enhancements for channels other than PUSCH and PUCCH]

# 7	Conclusions

Annex <A>:
Simulation assumptions

# A.1	Simulation assumptions for FR1

This clause describes the link-level simulation assumptions for FR1.  Table A.1-1 shows the general parameters for all channels. Table A.1-2~Table A.1-8 shows the channel-specific parameters for each channel respectively.

Table A.1-1: General parameters for FR1

| Parameter | Value |
| --- | --- |
| Scenario and frequency | Urban: 4GHz (TDD), 2.6GHz (TDD) Rural: 4GHz (TDD), 2.6GHz (TDD), 2GHz (FDD), 700MHz (FDD) Rural with long distance: 700MHz (FDD), 4GHz (TDD) |
| Frame structure for TDD | DDDSU (S: 10D:2G:2U) only for 4GHz DDDSUDDSUU (S: 10D:2G:2U) only for 4GHz DDDDDDDSUU (S: 6D:4G:4U) only for 2.6GHz Other frame structures can be reported by companies. |
| Target data rates for eMBB | Urban: DL 10Mbps, UL 1Mbps Rural: DL 1Mbps, UL 100kbps Rural with long distance: DL 1Mbps, UL 100kbps, 30kbps (optional) |
| Packet size for VoIP | A packet size of [320] bits with 20ms data arriving interval is adopted. TBD: TBS for SIP invite message. Payload of 1500 bytes can be a starting point. |
| Latency requirements for VoIP | 50ms/100ms |
| Pathloss model (select from LoS or NLoS) | Urban: NLoS Rural: NLoS and LoS |
| BWP | 100MHz for 4GHz and 2.6GHz. 20MHz for 2GHz (FDD) 20MHz (optional for 10MHz) for 700MHz. (FDD) |
| Channel model for link-level simulation | TDL-C for NLOS, TDL-D for LOS. [CDL] |
| Delay spread | Urban: 300ns Rural: 300ns Rural with long distance: 30ns |
| UE velocity | Urban: 3km/h for indoor Rural: 3km/h for indoor, 120km/h (optional 30km/h) for outdoor |
| Number of antenna elements for BS | - Urban: 192 antenna elements for 4GHz and 2.6GHz, (M,N,P,Mg,Ng) = (12,8,2,1,1) (optional) 128 antenna elements for 4GHz, (M,N,P,Mg,Ng) = (8,8,2,1,1) - Rural: 64 antenna elements for 4GHz and 2.6GHz (M,N,P,Mg,Ng) = (8,4,2,1,1) 32 antenna elements for 2GHz (M,N,P,Mg,Ng) = (8,2,2,1,1) 16 antenna elements for 700MHz (M,N,P,Mg,Ng) = (4,2,2,1,1) |
| Number of TxRUs for BS | gNB architectures to study: - 2 or 4 TXRUs for 2GHz, 700 MHz - 64TxRUs for 2.6 and 4 GHz. - Optional: 32 TXRUs at 2 GHz gNB modeling in LLS for TDL: - Option 1: 2 or 4 gNB receive chains in LLS. FFS: correlation - Option 2: Number of gNB receive chains = number of TXRUs in LLS. FFS: correlation [gNB architectures to study for CDL: - Urban: 64 receive chains for 2.6 and 4 GHz in LLS - Rural: 8 receive chains for 4GHz and 2.6GHz in LLS - 4 receive chains for 2GHz and 700MHz in LLS.] [gNB modeling in LLS for CDL: Number of gNB receive chains = number of TXRUs in LLS.] |

Table A.1-2: Channel-specific parameters for PUSCH for FR1

| Parameter | Value |
| --- | --- |
| Frequency hopping | w/ or w/o frequency hopping |
| BLER | For eMBB, w/ HARQ, 10% iBLER; w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| Number of UE transmit chains | 1, 2 (optional) |
| DMRS configuration | For 120km/h, (Optional: 30km/h): Type I, 2 or 3 DMRS symbol, no multiplexing with data. For frequency hopping: Type I, 1 or 2 DMRS symbol for each hop, no multiplexing with data. PUSCH mapping Type and DMRS position are reported by companies. Working assumption: For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. |
| Waveform | DFT-s-OFDM, CP-OFDM (optional) |
| SCS | 30kHz for TDD, 15kHz for FDD. |
| PUSCH duration | 14 OS |
| Repetitions | For eMBB, w/o repetition as baseline, w/ repetition (optional). For VoIP, w/ repetition. The actual number of repetitions is reported by companies. FFS: Repetition type B |
| HARQ configuration | For eMBB, whether HARQ is adopted is reported by companies. For VoIP, w/ HARQ. The maximum number of HARQ transmission (limited by frame structure and latency requirements) can be reported by companies. |
| PRBs/TBS/MCS for eMBB | Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. Companies are encouraged to use 30 PRBs for 1Mbps, 4 PRBs for 100kbps, 1 PRB for 30kbps as a starting point. TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. |
| PRBs/MCS for VoIP | [4 PRBs] for VoIP as starting point. Other values of PRBs can be reported by companies. QPSK, pi/2 BPSK (optional) |

Table A.1-3: Channel-specific parameters for PUCCH for FR1

| Parameter | Value |
| --- | --- |
| PUCCH format | Format 1, 2bits UCI. Format 3, [4bits (3 bits A/N + 1 bit SR)]/11/22 bits UCI |
| Frequency hopping | w/ frequency hopping |
| BLER | - For PUCCH format 1: DTX to ACK probability: 1%. NACK to ACK probability: 0.1%. ACK missed detection probability: 1%. - For PUCCH format 3: BLER for Ack/Nack, SR: 1% FFS: BLER for CSI (10% or 1%) |
| Number of UE transmit chains | 1 |
| DMRS configuration | FFS: number of DMRS symbols for PUCCH Format 3. |
| SCS | 30kHz for TDD, 15kHz for FDD. |
| Repetitions | w/ repetition (optional), w/o repetition for PUCCH. The maximum number of repetitions is 8. |
| PUCCH duration | 14 OS |
| Number of PRBs | 1 PRB |

Table A.1-4: Channel-specific parameters for PRACH for FR1

| Parameter | Value |
| --- | --- |
| Format | Format 0, Format B4, or Format C2 |
| SCS | Reported by companies. |
| Performance metric | 1% missed detection at 0.1% false alarm probability FFS: 10% missed detection. |
| Number of UE transmit chains | 1, 2 (optional) |
| Other parameters | Reported by companies. |

Table A.1-5: Channel-specific parameters for PUSCH of Msg.3 for FR1

| Parameter | Value |
| --- | --- |
| Frequency hopping | w/ or w/o frequency hopping |
| Number of UE transmit chains | 1, 2 (optional) |
| Number of DMRS symbol | w/o frequency hopping: 3, w/ frequency hopping: 2 for each hop |
| Waveform | DFT-s-OFDM |
| SCS | 30kHz for TDD, 15kHz for FDD. |
| HARQ configuration | For eMBB, whether HARQ is adopted is reported by companies. For VoIP, w/ HARQ. The maximum number of HARQ transmission (limited by frame structure and latency requirements) can be reported by companies. |
| PUSCH duration | 14 OS |
| Number of PRBs | 2 |
| TBS | 56 bits |
| Other parameters | Reported by companies. |

Table A.1-6: Channel-specific parameters for PDSCH for FR1

| Parameter | Value |
| --- | --- |
| BLER | For eMBB, w/ HARQ, 10% iBLER; w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| Waveform | CP-OFDM |
| Number of UE receive chains | 4 for 4GHz/2.6GHz, 2 or 4 for 2GHz, 2 for 700MHz |
| SCS | 30kHz for TDD, 15kHz for FDD. |
| HARQ configuration | For eMBB, whether HARQ is adopted is reported by companies. For VoIP, w/ HARQ. The maximum number of HARQ transmission (limited by frame structure and latency requirements) can be reported by companies. |
| DMRS configuration | 3 DMRS symbols is used for PDSCH of Msg.2. For 120km/h, (Optional: 30km/h): Type I, 2 or 3 DMRS symbol, no multiplexing with data. For frequency hopping: Type I, 1 or 2 DMRS symbol for each hop, no multiplexing with data. PDSCH mapping Type and DMRS position are reported by companies. Working assumption: For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. |
| PRBs/MCS/TBS | Reported by companies. |
| PDSCH duration | 12 OS For PDSCH of Msg.4, [12 OS] |
| Payload size for PDSCH of Msg.4 | FFS: [3000bits] |
| Other parameters | Reported by companies. |

Table A.1-7: Channel-specific parameters for PDCCH for FR1

| Parameter | Value |
| --- | --- |
| Number of UE receive chains | 4 for 4GHz/2.6GHz, 2 or 4 for 2GHz, 2 for 700MHz |
| SCS | 30kHz for TDD, 15kHz for FDD. |
| Aggregation level | 16 |
| Payload | 40 bits |
| CORESET size | 2 symbols, 48 PRBs |
| Tx Diversity | Reported by companies |
| BLER | 1% BLER FFS: 10% BLER |
| Number of SSB for broadcast PDCCH of Msg.2 | Reported by companies |
| Other parameters | Reported by companies |

Table A.1-8: Channel-specific parameters for SSB for FR1

| Parameter | Value |
| --- | --- |
| Number of UE receive chains | 4 for 4GHz/2.6GHz, 2 or 4 for 2GHz, 2 for 700MHz |
| SCS | 30kHz for TDD, 15kHz for FDD. |
| Periodicity | 20ms |
| Performance metric | Combination of 4 SSBs in 80ms. Note: UE is not assumed to know the SS/PBCH block index |
| Other parameters | Reported by companies. |

# A.2	Simulation assumptions for FR2

This clause describes the link-level simulation assumptions for FR2.  Table A.2-1 shows the general parameters for all channels. Table A.2-2~Table A.2-8 shows the channel-specific parameters for each channel respectively.

Table A.2-1: General parameters for FR2

| Parameter | Value |
| --- | --- |
| Scenario and frequency | Indoor: 28GHz (TDD) Urban: 28GHz (TDD) Suburban: 28GHz (TDD) |
| Frame structure for TDD | DDDSU (S: 10D:2G:2U) DDSU (S: 11D:3G:0U) Other frame structures can be reported by companies. |
| Target data rates for eMBB | Indoor: DL: 25Mbps, UL:5Mbps Urban: DL: 25Mbps, UL: 5Mbps Suburban: FFS: (DL: 1Mbps, UL: 50kbps) |
| Packet size for VoIP | A packet size of [320] bits with 20ms data arriving interval is adopted. TBD: TBS for SIP invite message. Payload of 1500 bytes can be a starting point. |
| Latency requirements for VoIP | 50ms/100ms |
| BWP | 100MHz, [400MHz] |
| Channel model for link-level simulation | CDL- A, TDL-A, [urban/suburban: TDL-C] Note: company can provide simulation results based on either TDL channel or CDL model |
| Delay spread | Indoor scenario: 30ns Urban scenario: 100ns Suburban scenario: 100ns |
| UE velocity | Indoor scenario:3km/h Urban scenario: 3km/h for indoor, 30km/h for outdoor. Suburban scenario: 3km/h for indoor, 30km/h, (optional: 120km/h) for outdoor. |
| Number of antenna elements for BS | Indoor scenario: 128 (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1) Urban/suburban scenario: 256, (M,N,P,Mg,Ng) = (4, 8, 2, 2, 2) Optional: 512, (M,N,P,Mg,Ng) = (8,8,2,2,2) |
| Number of TxRUs for BS | 2 Note: Analog beamforming is assumed. |
| Number of UE antenna elements | 8, one panel:(M, N, P) = (2,2,2), FFS: Two panels in link budget, one panel in LLS, 16 for each panel: (M, N, P) = (4,2,2) |

Table A.2-2: Channel-specific parameters for PUSCH for FR2

| Parameter | Value |
| --- | --- |
| Frequency hopping | w/ or w/o frequency hopping |
| BLER | For eMBB, w/ HARQ, 10% iBLER, Optional: companies report rBLER. w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| Number of UE Tx/Rx chains | 1T2R, 2T2R |
| DMRS configuration | For 30km/h (optional: 120km/h): Type I, 2 or 3 DMRS symbol, no multiplexing with data. For frequency hopping for PUSCH: Type I, 1 or 2 DMRS symbol for each hop, no multiplexing with data. PUSCH/PDSCH mapping Type and DMRS position are reported by companies. Working assumption: For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. |
| Waveform | DFT-s-OFDM FFS: CP-OFDM |
| SCS | 120kHz. |
| PUSCH duration | 14 OS |
| Repetitions | For eMBB, w/o repetition as baseline, w/ repetition (optional). For VoIP, w/ repetition. The actual number of repetitions is reported by companies. FFS: Repetition type B for PUSCH |
| HARQ configuration | For eMBB, whether HARQ is adopted is reported by companies. For VoIP, w/ HARQ. The maximum number of HARQ transmission (limited by frame structure and latency requirements) can be reported by companies. |
| PRBs/TBS/MCS for eMBB | Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. Companies are encouraged to use [30] PRBs for 5Mbps for PUSCH as a starting point. TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. |
| PRBs/MCS for VoIP | [4 PRBs] for VoIP as starting point. Other values of PRBs can be reported by companies. QPSK for PUSCH Optional: pi/2 BPSK for PUSCH |

Table A.2-3: Channel-specific parameters for PUCCH for FR2

| Parameter | Value |
| --- | --- |
| PUCCH format | Format 1, 2bits UCI. Format 3, [4bits (3 bits A/N + 1 bit SR)]/11/22 bits UCI FFS: Format 0, 2 |
| Frequency hopping | w/ frequency hopping |
| BLER | - For PUCCH format 1: DTX to ACK probability: 1%. NACK to ACK probability: 0.1%. ACK missed detection probability: 1%. - For PUCCH format 3: BLER for Ack/Nack, SR: 1% FFS: BLER for CSI (10% or 1%) |
| Number of UE transmit chains | 1 |
| DMRS configuration for | FFS: [4] DMRS symbols for PUCCH Format 3. |
| SCS | 120kHz |
| Repetitions | w/ repetition (optional), w/o repetition for PUCCH. The maximum number of repetitions is 8. |
| PUCCH duration | 14 PFDM symbols FFS: 4 OFDM symbols |
| Number of PRBs | 1 PRB |

Table A.2-4: Channel-specific parameters for PRACH for FR2

| Parameter | Value |
| --- | --- |
| Format | Format B4, (Optional: Format C2) |
| SCS | Reported by companies. |
| Performance metric | 0.1% false alarm, 1% miss-detection FFS: 10% missed detection. |
| Number of UE Tx chains | 1T, 2T |
| Number of SSB beams | Reported by companies |
| Other parameters | Reported by companies. |

Table A.2-5: Channel-specific parameters for PUSCH of Msg.3 for FR2

| Parameter | Value |
| --- | --- |
| Frequency hopping | w/ or w/o frequency hopping |
| Number of UE Tx chains | 1T, 2T |
| SCS | 120kHz |
| HARQ configuration | For eMBB, whether HARQ is adopted is reported by companies. For VoIP, w/ HARQ. The maximum number of HARQ transmission (limited by frame structure and latency requirements) can be reported by companies. |
| TBS | 56 bits |

Table A.2-6: Channel-specific parameters for PDSCH for FR2

| Parameter | Value |
| --- | --- |
| BLER | For eMBB, w/ HARQ, 10% iBLER, Optional: companies report rBLER. w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| Waveform | CP-OFDM |
| Number of UE receive chains | 2 |
| SCS | 120kHz |
| HARQ configuration | For eMBB, whether HARQ is adopted is reported by companies. For VoIP, w/ HARQ. The maximum number of HARQ transmission (limited by frame structure and latency requirements) can be reported by companies. |
| DMRS configuration | For 30km/h (optional: 120km/h): Type I, 2 or 3 DMRS symbol, no multiplexing with data. For frequency hopping for PUSCH: Type I, 1 or 2 DMRS symbol for each hop, no multiplexing with data. PUSCH/PDSCH mapping Type and DMRS position are reported by companies. Working assumption: For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. |
| PRBs/TBS/MCS for eMBB | Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. Companies are encouraged to use full bandwidth for 25Mbps for PDSCH as a starting point. TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. |
| PRBs/MCS for VoIP | [4 PRBs] for VoIP as starting point. Other values of PRBs can be reported by companies. QPSK for PDSCH |
| PDSCH duration | 12 OS |

Table A.2-7: Channel-specific parameters for PDCCH for FR2

| Parameter | Value |
| --- | --- |
| Number of UE receive chains | 2 |
| SCS | 120kHz |
| Aggregation level | 16 |
| Payload | 40 bits |
| CORESET size | 2 symbols, 48 PRBs |
| Tx Diversity | Reported by companies |
| BLER | 1% BLER FFS: 10% BLER |
| Number of SSB for broadcast PDCCH of Msg.2 | Reported by companies |
| Other parameters | Reported by companies |

Table A.2-8: Channel-specific parameters for SSB for FR2

| Parameter | Value |
| --- | --- |
| Number of UE Tx/Rx chains | 1T2R, 2T2R |
| SCS | 120kHz |

# A.3	Link budget template

Annex <B> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-05 | RAN1#101-e | R1-2004753 |  |  |  | Skeleton TR | 0.0.1 |
| 2020-08 | RAN1#102-e | R1-2005730 |  |  |  | Inclusion of agreements made at RAN1#101-e on evaluation methodology and simulation assumptions | 0.0.2 |