| 3GPP TR 38.838 V0.2.0 (2021-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on XR (Extended Reality) Evaluations for NR (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Table of Contents

1	Foreword	9

2	Scope	10

3	References	10

4	Definitions of terms, symbols and abbreviations	11

4.1	Terms	11

4.2	Symbols	11

4.3	Abbreviations	11

5	Introduction	12

6	Traffic Models	13

6.1	Generic DL Traffic Model	13

6.1.1	Single Stream DL Traffic Model	13

6.1.1.1	Packet Size	13

6.1.1.2	Packet Arrival	14

6.1.1.3	PDB	15

6.1.1.4	Packet Success Rate Requirement	15

6.1.1.5	Dual Eye Buffer Model	15

6.1.2	Multi-Streams DL Traffic Model	16

6.1.2.1	Option 1 (I+P)	16

6.1.2.2	Option 2 (video+audio/data)	17

6.1.2.3	Option 3 (FOV + omnidirectional view)	17

6.2	Generic UL Pose/Control Traffic	18

6.3	VR Traffic Model	18

6.3.1	VR DL Stream	18

6.3.2	VR UL Stream	19

6.4	CG Traffic Model	19

6.4.1	CG DL Stream	19

6.4.2	CG UL Stream	20

6.5	AR Traffic Model	20

6.5.1	AR DL Stream(s)	20

6.5.2	AR UL Stream(s)	20

6.5.2.1	Model 1 (one stream model)	21

6.5.2.2	Model 2 (two streams model)	21

6.5.2.3	Model 3A (three streams model A)	21

6.5.2.4	Model 3B (three streams model B)	22

7	Deployment Scenarios	22

8	XR Capacity Evaluation	23

8.1	Purpose of Study	23

8.2	KPI	23

8.2.1	UE Satisfaction	23

8.2.2	System Capacity	23

8.3	Capacity Results	23

8.3.1	Capacity baseline performance	23

8.3.1.1	FR1 DL	23

8.3.1.2	FR1 UL	31

8.3.1.3	FR2 DL	36

8.3.1.4	FR2 UL	43

8.3.2	Capacity Comparison for Different Parameters/Configurations	46

8.3.2.1	Capacity Comparison for Different Data-rate	46

8.3.2.2	Capacity Comparison for Different PDB/PER Values	49

8.3.2.3	Impact of Jitter on Capacity	58

8.3.2.4	Impact of Dual-eye Buffers Staggering	59

8.3.2.5	Impact of TDD Frame Format	61

8.3.2.6	Impact of Bandwidth	64

8.3.2.7	Impact of FDM/SDM and mini-slot	66

8.3.3	Potential Capacity Enhancements	67

8.3.3.1	Staggering of packet arrivals at gNB among UEs	67

8.3.3.2	Delay Aware Scheduler	72

8.3.3.3	Frame Level Integrated Transmission Scheduler	74

8.3.3.4	Cooperative MIMO/Precoding via Bi-directional Training (BiT)	75

8.3.3.5	Network Coding (NC)/Outer Coding (OC)	81

8.3.3.6	gNB Scheduling Awareness UE Playout Buffer	82

8.3.3.7	Impact of Carrier Aggregation	83

8.3.3.8	Prioritizing important stream	83

8.3.3.9	Adaptive Inter-UE/Intra-UE Multiplexing Techniques	87

8.3.3.10	HARQ-ACK enhancement for DG scheduling	89

8.3.3.11	Enhanced buffer status reporting for UL transmission	90

8.3.3.12	Application Data Unit (ADU) dropping	92

9	XR UE Power Consumption Evaluation	92

9.1	Purpose of Study	92

9.2	KPI	93

9.3	UE Power Consumption Evaluation	93

9.3.1	Baseline Power Evaluation Results	93

9.3.1.1	FR1	93

9.3.1.2	FR2	124

9.3.2	Performance Comparison for Parameters/Modelling	134

9.3.2.1	Trade-off between Capacity and Power	134

9.3.2.2	Performance Comparison for Different DL Frame Generation Rates	134

9.3.2.3	Performance Comparison for Different Data Rates	135

9.3.2.4	Performance Comparison for Different Pose Periodicity	135

9.3.3	Potential Enhancements	136

9.3.3.1	Enhanced CDRX	136

9.3.3.2	Jitter Handling	148

9.3.3.3	XR dedicated PDCCH Monitoring Window	158

9.3.3.4	Additional Packet Delay Budget with Playout Buffer	158

9.3.3.5	Traffic Arrival Offset Staggering	159

9.3.3.6	SR group switching	161

9.3.3.7	UL active time	162

9.3.3.8	Enhanced PDCCH monitoring	163

10	XR Coverage Evaluation	164

10.1	Purpose of Study	164

10.2	KPI	164

10.3	XR Coverage Evaluation	164

10.3.1	Coverage based on Methodology 1	164

10.3.1.1	FR1	164

10.3.1.2	FR2	169

10.3.2	Coverage based on Methodology 2	170

10.3.2.1	FR1	170

10.3.2.2	FR2	173

11	XR Mobility evaluations	173

11.1	Purpose of study	173

11.2	KPI	173

11.3	Mobility evaluation results	173

11.3.1	Consecutive XR packets lost due to a HO event, N	174

11.3.2	Minimum target time interval between HO events, T	175

12	Conclusions	178

Annex <A>: Evaluation Methodology	181

A.1	Evaluation Methodology for Capacity	181

A.2	Evaluation Methodology for Power	186

A.3	Evaluation Methodology for Coverage	188

A.4	Evaluation Methodology for Mobility	189

Annex <B> Source Specific Capacity Performance Evaluation Results	190

B.1.	FR1 DL	190

B.1.1.	DU Scenario	190

B.1.1.1.	VR/AR	190

B.1.1.2.	CG	202

B.1.2.	InH Scenario	205

B.1.2.1.	VR/AR	205

B.1.2.2.	CG	209

B.1.3.	Uma Scenario	211

B.1.3.1.	VR/AR	211

B.1.3.2.	CG	214

B.2.	FR1 UL	216

B.2.1.	DU Scenario	216

B.2.1.1.	VR/CG (Pose/control-stream)	216

B.2.1.2.	AR (1 stream: Scene/video/data/voice-stream)	216

B.2.1.3.	AR (2 streams: Pose/control-stream + scene/video/data/voice-stream)	218

B.2.1.4.	AR (3 streams: Video stream+Data/audio stream+Pose/control stream)	219

B.2.1.5.	AR (3 streams: Pose/control-stream + I/P-stream)	219

B.2.2.	InH Scenario	219

B.2.2.1.	VR/CG (Pose/control-stream)	219

B.2.2.2.	AR (1 stream: Scene/video/data/voice-stream)	220

B.2.2.3.	AR (2 streams: Pose/control-stream + scene/video/data/voice-stream)	221

B.2.2.4.	AR (3 streams: Video stream+Data/audio stream+Pose/control stream)	221

B.2.3.	Uma Scenario	222

B.2.3.1.	VR/CG (Pose/control-stream)	222

B.2.3.2.	AR (1 stream: Scene/video/data/voice-stream)	222

B.2.3.3.	AR (2 streams: Pose/control-stream + scene/video/data/voice-stream)	223

B.3.	FR2 DL	224

B.3.1.	DU Scenario	224

B.3.1.1.	VR/AR	224

B.3.1.2.	CG	227

B.3.2.	InH Scenario	228

B.3.2.1.	VR/AR	228

B.3.2.2.	CG	232

B.4.	FR2 UL	233

B.4.1.	DU Scenario	233

B.4.1.1.	VR/CG (Pose/control-stream)	233

B.4.1.2.	AR (1 stream: Scene/video/data/voice-stream)	233

B.4.1.3.	AR (2 streams: Pose/control-stream + scene/video/data/voice-stream)	234

B.4.2.	InH Scenario	235

B.4.2.1.	VR/CG (Pose/control-stream)	235

B.4.2.2.	AR (1 stream: Scene/video/data/voice-stream)	235

B.4.2.3.	AR (2 streams: Pose/control-stream + scene/video/data/voice-stream)	236

Annex <C> Source Specific Power Performance Evaluation Results	237

Annex <D> Source Specific Mobility Performance Evaluation Results	237

B.5.	Consecutive XR packets lost due to a HO event, N	237

B.6.	Minimum target time interval between HO events, T	244

Annex <E> (informative):	265

Change history	265

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Scope

The present document captures the results and findings from the study item "Study on XR Evaluation for NR "[2].

The purpose of this TR is to document the evaluation methodology for XR evaluation including XR applications, simulation scenarios, traffic models, KPIs, simulation parameters, etc,

to document the performance evaluation results of XR applications in NR for both FR1 and FR2 considering the scenarios and services of interest,

to document the identified problems/challenges in supporting XR applications of interest in various scenarios.

This activity involves the Radio Access work area of the 3GPP studies and has potential impacts both on the Mobile Equipment and Access Network of the 3GPP systems.

# References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

- 3GPP TR 21.905: "Vocabulary for 3GPP Specifications"

- 3GPP RP-201145: "Revised SI on XR Evaluations for XR"

- 3GPP R1-2104023: “LS on Status Update on XR Traffic”

- 3GPP S4-210614: “FS_XRTRaffic: Permanent document, v0.6.0”

- 3GPP TR 23.501: “System architecture for the 5G System (5GS)”

- 3GPP TR 38.840: “Study on User Equipment (UE) power saving in NR”

- 3GPP R1-2101765, “LS on XR-Traffic Models”

# Definitions of terms, symbols and abbreviations

## Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ACK	Acknowledgement

BWP	Bandwidth Part

CSI	Channel State Information

DL	Downlink

DMRS	Dedicated Demodulation Reference Signals

FDD	Frequency Division Duplex

gNB	NR Node B

fps	Frames per second

HARQ	Hybrid Automatic Repeat reQuest

iBLER	initial BLock Error Rate

MCS	Modulation and Coding Scheme

NACK	Negative Acknowledgement

PDCCH	Physical Downlink Control Channel

PDB	Packet Delay Budget

PSG	Power Saving Gain

PSR 	Packet Success Rate

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

PDSCH	Physical Downlink Shared Channel

SR	Scheduling Request

STD	STandard Deviation

TDD	Time Division Duplex

UE	User Equipment

UL	Uplink

XR 	Extended Reality

# Introduction

XR Applications

eXtended Reality(XR) is a term for different types of realities and refers to all real-and-virtual combined environments and human-machine interactions generated by computer technology and wearables. It includes following representative forms and the areas interpolated among them. [to add a few sentences explaining CG]

- Augmented Reality (AR)

- Mixed Reality (MR)

- Virtual Reality (VR)

XR and Cloud Gaming (CG) are currently one of the most important 5G media applications under consideration in the industry[2].

[System Architecture

One specific aspect to be considered is the role of Edge Computing as a network architecture to enable XR and Cloud Gaming. Edge Computing is a concept that enables cloud computing capabilities and service environments to be deployed close to the cellular network. It promises several benefits such as lower latency, higher bandwidth, reduced backhaul traffic and prospects for several new services as indicated in the SA6 Study on application architecture for enabling Edge Applications (TR 23.758). Edge Applications are expected to take advantage of the low latencies enabled by 5G and the Edge network architecture to reduce the end-to-end Application-level latencies. Edge Computing is a valuable enabler which should be considered to help 5G systems achieve the required performance to enable XR and Cloud Gaming[2].] This part may be softened/removed per further discussion.

Traffic Characteristics

5G NR is designed to support applications demanding high throughput and low latency in line with the requirements posed by the support of XR and Edge Computing applications in NR networks. XR and Edge Computing are services enabled by Rel-15 NR networks[2].

Objective

The objectives of this study are as follows.

- Confirm XR and Cloud Gaming applications of interest

- Identify the traffic model for each application of interest taking outcome of SA WG4 work as input, including considering different upper layer assumptions, e.g. rendering latency, codec compression capability etc.

- Identify evaluation methodology to assess XR and CG performance along with identification of KPIs of interest for relevant deployment scenarios

- Once traffic model and evaluation methodologies are agreed, carry out performance evaluations towards characterization of identified KPIs

# Traffic Models

In this section, we provide the DL and UL traffic models for VR, CG, and AR applications. Since DL/UL traffic models for these applications share similar characteristics, we first define a generic and parameterized DL / UL traffic model, which could be later used in defining VR, CG, AR applications.

The traffic model defined in this section is statistical traffic model, where packet size and packet arrival process are characterized by certain random variables. The described model is based on the input XR traffic study from SA4[7][3][4].

## Generic DL Traffic Model

### Single Stream DL Traffic Model

This section provides a parameterized generic single stream DL traffic model. In this model, as shown in Figure 1, the XR DL traffic is modelled as a sequence of video frames arriving at gNB according to the considered video frame rates and random jitter. The size of each frame is also random according to a certain distribution.

Figure 1 Single stream DL Traffic Model

#### Packet Size

In this model, a packet models the set of IP packets belong to the same video frame. The video frame includes both left and right eye frame sharing the same buffer, which is referred to as ‘single stream for dual eye buffer’ or ‘single eye buffer’ throughout this document.

The size of a packet is determined by the given data rates and frame rates, which is modelled as a random variable following truncated Gaussian distribution with following statistical parameters.

Table 1 Statistical parameters for packet size following truncated Gaussian distribution

| Parameter | unit | Baseline values for evaluation | Optional values for evaluation for single eye buffer |
| --- | --- | --- | --- |
| Mean: M | byte | R×1e6 / F / 8 | R×1e6 / F / 8 |
| STD | byte | 10.5% of M | 3 % of M |
| Max | byte | 150% of M | 109% of M |
| Min | byte | 50% of M | 91% of M |
| R: data rate of the flow in Mbps. F: frame generation rate of the flow in fps. Note that the mean and STD are for before truncation applies. Note that the value of R, F depend on application. |  |  |  |

Exploration to other distributions for packet size are left up to each company and could be reported with the modelling details.

#### Packet Arrival

In this model, the packet arrival rate is determined by the frame generation rate, e.g., 60fps. Accordingly, the average packet arrival periodicity is given by the inverse of the frame rate, e.g., 16.6667ms = 1/60fps. The periodic arrival without jitter gives the arrival time at gNB for packet with index k (=1,2,3….) as

k/X*1000 [ms],

where X is the given frame generation rates (per second).

Note that this periodic packet arrival implicitly assumes fixed delay contributed from network side including fixed video encoding time, fixed network transfer delay, etc.

However, in a real system, the varying frame encoding delay and network transfer time introduces jitter in packet arrival time at gNB which. In this model, the jitter is modelled as a random variable added on top of periodic arrivals. The jitter follows truncated Gaussian distribution with following statistical parameters shown in Table 2.

Table 2 Statistical parameters for jitter

| Parameter | unit | Baseline value for evaluation | Optional value for evaluation |
| --- | --- | --- | --- |
| Mean | ms | 0 |  |
| STD | ms | 2 |  |
| Truncation range | ms | [-4, 4] | [-5, 5] |

Note that the given parameter values and considered frame generation rates (60 or 120 in this model) ensure that packet arrivals are in order (i.e., arrival time of a next packet is always larger than that of the previous packet).

Thus, the periodic arrival with jitter gives the arrival time for packet with index k (=1,2,3….) as

offset + k/F*1000 + J [ms],

where F is the given frame generation rates (per second) and J is a random variable capturing jitter. Note that actual traffic arrival timing of traffic for each UE could be shifted by the UE specific arbitrary offset.

#### PDB

The latency requirement of XR traffic in RAN side (i.e., air interface) is modelled as packet delay budget (PDB). The PDB is a limited time budget for a packet to be transmitted over the air from a gNB to a UE.

For a given packet, the delay of the packet incurred in air interface is measured from the time that the packet arrives at the gNB to the time that it is successfully transferred to the UE. If the delay is larger than a given PDB for the packet, then, the packet is said to violate PDB, otherwise the packet is said to be successfully delivered.

The value of PDB may vary for different applications and traffic types.

#### Packet Success Rate Requirement

The performance requirement in terms of packet success rate is given as X (%). If packet delivery delay exceed a given PDB, then, the packet is counted as failure. Following values for packet success rate X are considered.

Table 3 Packet Success Rate Requirement

| Parameter | unit | Baseline values for evaluation | Optional values for evaluation |
| --- | --- | --- | --- |
| Packet success rate requirement X for DL single stream | % | 99 | 95, 99.99, etc |

Note that the Packet error rate (PER) in percentage is given as PER = 100 – X.

#### Dual Eye Buffer Model

This section describes optional modification of packet size and frame rates for separate packet arrival for dual-eye buffer.

In single eye buffer model, the frame for both eyes arrive at the same time as a single packet. Thus, mean packet size M is given as R×1e6 / F, where R is frame generation rate in Mbps and F is frame generation rate.

Whereas, in dual eye buffer model of data rate R, the left and right eye frame arrive separately with a time offset, which makes the arrival process effectively equivalent to have two times of frame rates and half mean packet size of that of single eye buffer model. Accordingly, we have mean packet size M of dual eye buffer model is given as R×1e6 / (2×F) for dual eye buffer model.

Table 4 Statistical parameter values for dual eye buffer packet size

| Parameter | unit | values for evaluation | Optional values for evaluation |
| --- | --- | --- | --- |
| Mean: M | byte | R×1e6 / (2×F) /8 | R×1e6 / (2×F) / 8 |
| STD | byte | 10.5% of M | 4% of M |
| Max | byte | 150% of M | 112% of M |
| Min | byte | 50% of M | 88% of M |
| R: data rate of the flow in Mbps F: frame generation rate of the flow in fps |  |  |  |

### Multi-Streams DL Traffic Model

This section provides optional multi-streams model for XR DL traffic.

- Option 1: I-frame + P-frame

  - Option 1A: slice-based traffic model

  - Option 1B: Group-Of-Picture (GOP) based traffic model

- Option 2: video + audio/data

- Option 3: FOV + omnidirectional stream

#### Option 1 (I+P)

For Option 1, two streams (I-stream and P-stream) are modelled according to following table.

- Stream 1: I stream

- Stream 2: P stream

Depending on the video encoding scheme, two additional sub models – slice based, and Group of Picture (GOP)-based models are defined.

- Slice-based: In this encoding scheme, a single video frame is divided into N slices. Out of N, one slice is I slice and remaining N-1 slices are P slices. N packets (one I and N-1 P) packets corresponds to one video frame arriving at the same time.

- GOP-based: In this encoding scheme, a single video frame is either I frame or P frame. I frame is transmitted every K frames, where K is the GOP size, i.e., every group of picture. One video frame arrives at a time as a packet.

Table 5 Statistical parameters for Option 1 multi streams DL traffic model

| Two data streams | Option 1A: slice-based |  | Option 1B: GOP-based |  |  |
| --- | --- | --- | --- | --- | --- |
|  | I-stream | P-stream | I-stream |  | P-stream |
| Packet modelling | Slice-level |  | Frame-level |  |  |
| Traffic pattern | Both streams are periodic at 60 fps with the same jitter model as for single stream. |  | Follow the GOP structure, where GOP size K = 8 with the same jitter model as for single stream. |  |  |
| Number of packets per stream at a time | 1 | N-1 | I-frame: 1 or 0 P-frame: 0 or 1 At each time instant, there is either only one I-stream packet or only one P-stream packet |  |  |
|  | N = 8: the number of slices per frame. |  |  |  |  |
| Average data rate per stream | RI=R*αN-1+α | RP=R*N-1N-1+α | RI=R*αK-1+α |  | RP=R*K-1K-1+α |
|  | - R: average data rate of a single stream video - α: average size ratio between one I-frame/slice and one P-frame/slice - α = 1.5, 2 (baseline) - α = 3 (optional) |  |  |  |  |
| Packet size distribution | Truncated Gaussian distribution |  |  |  |  |
|  | Mean = RIFPS | Mean = RPFPS*(N-1) | Mean = RI*KFPS | Mean = RP*KFPS*(K-1) |  |
|  | - [STD, Max, Min]: [10.5, 150, 50]% of Mean packet size - FPS is the frame rate of the single stream video |  |  |  |  |
| Packet Success Rate X | Depends on application, see 6.3.1, 6.4.1, 6.5.1 for VR, CG, AR respectively. |  |  |  |  |
| PDB | Depends on application, see 6.3.1, 6.4.1, 6.5.1 for VR, CG, AR respectively. |  |  |  |  |

#### Option 2 (video+audio/data)

For Option 2, two streams (video + audio/data) are modelled.

- Stream 1: video

- Stream 2: audio/data

The stream 1 - video stream follows the generic single stream model given in section 6.1.1. The stream 2 - audio/data a periodic traffic with following parameters.

Table 6 Statistical parameter values for Option 2 multi streams model

| Parameters | unit | Baseline values for evaluation | Optional values for evaluation |
| --- | --- | --- | --- |
| Periodicity P | ms | 10 |  |
| Data rate: R | Mbps | 0.756, 1.12 |  |
| Packet size | byte | R×1e6 × P /1000 / 8 |  |
| PDB | ms | 30 | Other values can be optionally evaluated |
| Packet Success Rate | % | 99 | 99.9 |

#### Option 3 (FOV + omnidirectional view)

For Option 3, following two streams are modelled.

- Stream 1: FOV

- Stream 2: omnidirectional view stream

The detailed modelling of the two streams is left to company with the report of evaluation results.

## Generic UL Pose/Control Traffic

In this section, we provide the generic UL pose/control stream traffic model. A packet for UL pose/control arrives at UE periodically with following parameters.

Table 7 Statistical parameters for the UL pose/control traffic

| Parameters | unit | Baseline values for evaluation | Optional value for evaluation |
| --- | --- | --- | --- |
| Periodicity | ms | 4 | Other values can be optionally evaluated. |
| Jitter | ms | No jitter |  |
| Packet size | byte | 100 |  |
| PDB | ms | 10 |  |
| Packet Success Rate X | % | 99 | 90, 95 |

## VR Traffic Model

### VR DL Stream

Single Stream Model

The VR DL single stream follows generic single stream DL video traffic model in section 6.1.1 with following parameters.

Table 8 Statistical Parameters for single stream DL VR Traffic Model

| Parameters | unit | Baseline values for evaluation | Optional values for evaluation |
| --- | --- | --- | --- |
| data rate: R | Mbps | 30, 45 | 60 |
| frame generation rate: F | fps or Hz | 60 |  |
| PDB | ms | 10 | 5, 20 |

Optionally, following combination of packet success rate X and PDB could be also considered for evaluation.

Table 9 Optional (X, PDB) for single stream DL VR Traffic Model

| Parameter | unit | Optional values for evaluation |
| --- | --- | --- |
| Packet success rate requirement X and PDB pair (X, PDB) for DL single stream | (%, ms) | (99, 7), (95, 13) for VR/AR |

Multi-streams Model

The VR DL multi-streams follows generic multi-streams DL traffic model given in section 6.1.2 with following parameters.

Table 10 Statistical Parameters for multi streams DL VR Traffic Model

| Parameters | unit | Baseline values for evaluation | Optional values for evaluation |
| --- | --- | --- | --- |
| Packet Success rate X for I stream | % | 99 | Other values can be optionally evaluated. |
| Packet Success rate X for P stream | % | 99 | Other values can be optionally evaluated. |
| PDB for I stream | ms | 10 | Other values can be optionally evaluated. |
| PDB for P stream | ms | 10 | Other values can be optionally evaluated. |

For Option 2, two streams (video + audio/data) are modelled as given in Section 7.1.2.

- Stream 1: video

- Stream 2: audio/data

The stream 1 - video stream follows the generic single stream model given in section 6.1.1. The stream 2 - audio/data a periodic traffic with following parameters.

Table 11 Statistical parameter values for Option 2 multi streams model [“Option 2” to be clarified]

| Parameters | unit | Baseline values for evaluation | Optional values for evaluation |
| --- | --- | --- | --- |
| Periodicity P | ms | 10 |  |
| Data rate: R | Mbps | 0.756, 1.12 |  |
| Packet size | byte | R×1e6 × P /1000 / 8 |  |
| PDB | ms | 30 | Other values can be optionally evaluated |
| Packet Success Rate | % | 99 | 99.9 |

### VR UL Stream

VR UL Stream follows generic UL pose and control traffic model described in section 6.2.

## CG Traffic Model

### CG DL Stream

Single-stream Model

The CG DL stream follows generic single stream DL video traffic model in section 6.1.1 with following parameters.

Table 12 Statistical Parameters for single stream CG Traffic Model

| Parameters | unit | Baseline values for evaluation | Optional values for evaluation |
| --- | --- | --- | --- |
| data rate: R | Mbps | 30, 8 | 45 |
| frame generation rate: F | fps or Hz | 60 |  |
| PDB | ms | 15 | 10, 30 |

Optionally, following combination of X and PDB could be also considered for evaluation.

Table 13 Optional (X, PDB) pair for single stream CG Traffic Model

| Parameter | unit | Optional values for evaluation |
| --- | --- | --- |
| Packet success rate requirement X and PDB pair (X, PDB) for DL single stream | (%, ms) | (99, 12), (95, 18) for CG |

Multi-streams Model

The CG DL multi-streams follows generic multi-streams DL traffic model given in 6.1.2 with following parameters in.

Table 14 Statistical Parameters for multi streams DL CG Traffic Model

| Parameters | unit | Baseline values for evaluation | Optional values for evaluation |
| --- | --- | --- | --- |
| Packet Success rate X for I stream | % | 99 | Other values can be optionally evaluated. |
| Packet Success rate X for P stream | % | 99 | Other values can be optionally evaluated. |
| PDB for I stream | ms | 15 | Other values can be optionally evaluated. |
| PDB for P stream | ms | 15 | Other values can be optionally evaluated. |

### CG UL Stream

CG UL Stream follows generic UL pose and control traffic model described in section 6.2.

## AR Traffic Model

### AR DL Stream(s)

The AR DL Stream(s) has/have the same models as VR DL stream model given in section 6.3.1.

### AR UL Stream(s)

In this section, we provide four different options for AR UL traffic model. Given that AR has multiple streams in UL, one can choose a model from various options depending on what/how to model the streams. Four options are as follows.

- Model 1: one stream model

- Model 2: Two streams model

- Model 3A: Three streams model A

- Model 3B: Three streams model B

The detail of each model is given in following sections.

#### Model 1 (one stream model)

In Model 1, all AR UL flows are modelled as a single stream with following parameters.

Table 15 Statistical parameters for AR UL Model 1 (one stream model)

| Parameters | unit | value |
| --- | --- | --- |
| Packet size | byte | Follows section 6.3.1 (i.e., mean packet size = R×1e6 / F / 8, STD/Min/Max=10.5/50/150%) |
| packet generation rate: F | Hz | 60 |
| Jitter | ms | Optional, follows the description in 6.1.1.2 |
| Data rate: R | Mbps | 10 (baseline), 20 (optional) |
| PDB | ms | 30 (baseline), 10 or 15 or 60 (optional) |

Note that Model 1 is optional for power evaluation and baseline for capacity evaluation.

#### Model 2 (two streams model)

In Model 2, two streams are considered.

- Stream 1 for pose/control

  - Traffic model/requirement for stream 1 follows section 6.2.

- Stream 2 aggregating scene, video, data, and audio

  - Follows the statistical parameters shown in Table 15.

#### Model 3A (three streams model A)

In Model 3A, three steams are considered.

- Stream 1: pose/control

  - Traffic model/requirement for stream 1 follows section 6.2.

- Stream 2: A stream aggregating streams of scene and video

  - Follows the statistical parameters shown in Table 15.

- Stream 3: A stream aggregating streams of audio and data

Table 16 Statistical parameters for stream 3 of AR UL Model 3A (three streams model)

| Parameters | unit | value |
| --- | --- | --- |
| Data rate: R | Mbps | 0.756, 1.12 |
| Periodicity: P | ms | 10 |
| Packet size | byte | mean packet size = R×1e6 × P/1000 / 8 |
| PDB | ms | 30 |

#### Model 3B (three streams model B)

In Model 3B, three streams are considered

- Stream 1: pose/control

  - Traffic model/requirement for stream 1 follows section 6.2.

- Stream 2: I-stream for video

- Stream 3: P-stream for video

Table 17 Statistical Parameters for stream 2 and 3 of AR UL Model 3B (three streams model)

| Parameters | unit | Baseline values for evaluation | Optional values for evaluation |
| --- | --- | --- | --- |
| Packet Success rate X for I stream | % | 99 | Other values can be optionally evaluated. |
| Packet Success rate X for P stream | % | 99 | Other values can be optionally evaluated. |
| PDB for I stream | ms | 30 | Other values can be optionally evaluated. |
| PDB for P stream | ms | 30 | Other values can be optionally evaluated. |

Note: For stream 2 and stream 3, the I/P-stream model for DL video can be reused for UL video.  Companies should report detailed assumptions in their simulations on packet size distribution for each stream, packet arrival interval (or fps) for each stream, PDB for each stream, PER requirement for each stream, criteria to be satisfied UE.

# Deployment Scenarios

We consider following three different deployment scenarios for XR.

- Dense Urban: In this scenario, XR Ues are in urban area where gNBs are deployed densely with inter site distance (ISD) of 200m. User playing cloud gaming (CG), users experiencing VR/AR indoor and outdoor are considered. For FR1, 80/20% of Ues are assumed in indoor/outdoor. For FR2, 100% Ues are assumed to be outdoor.

- Indoor Hotspot: In this scenario, only indoor XR users are considered. VR or CG applications is more likely for indoor for work and gaming. Indoor AR application is also considered. This applies to both FR1 and FR2.

- Urban Macro: In this scenario, larger ISD of 500ms is considered, where XR users are distributed over larger area. Due to large ISD deployment, XR applications with lower rate would be more relevant to this scenario. Urban Macro scenario is evaluated for FR1 only.

# XR Capacity Evaluation

## Purpose of Study

In this section, we describe the KPI for capacity evaluations and provide evaluation results for capacity based on baseline parameters and optional parameters/modelling methods.

The purpose of capacity study is to understand the performance of NR systems for XR applications, and identify any issues and performance gaps, which could be useful for understanding the limitation of current NR systems in supporting XR applications and the potential directions for future necessary enhancements to better support XR.

## KPI

### UE Satisfaction

A UE is declared as a satisfied UE if all the considered streams meet their own PER and PDB requirements, i.e., more than a certain percentage of packets are successfully transmitted within a given air interface PDB. Specifically, we have followings depending on the evaluation directions considered.

- In DL-only evaluation, only DL streams are considered when identifying UE satisfaction.

- In UL-only evaluation, only UL streams are considered when identifying UE satisfaction.

### System Capacity

System capacity is identified as KPI for capacity study, which is defined as the maximum number of users per cell with at least Y % of UEs being satisfied.

- Y=90 (baseline) or 95 (optional)

- Other values of Y can also be evaluated optionally.

For details on how to evaluate capacity, see capacity evaluation section 0.

## Capacity Results

This section is to capture the evaluation results and the corresponding observations for capacity. The detailed evaluation results can be found in Annex B.

### Capacity baseline performance

#### FR1 DL

This section captures the capacity baseline performance evaluation results of FR1 DL.

Table 8.3.1.1-1. Summary of FR1 DL capacity evaluation results for single-stream

| Scenario | App | PDB | R | F(fps) | MIMO | Capacity result (UEs/cell) |  | Source | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | mean | range |  |  |
| DU | AR/VR | 10ms | 60Mbps | 60 | MU | 0 | 0 | Source 16 | Note 1, |
|  |  |  | 45Mbps | 60 | SU | 4.58 | 1.7~6 | Source 5, Source 7, Source 8, Source 9, Source 14, Source 16, Source 18 | Note 1 |
|  |  |  |  |  | SU | 4.77 | 4.1~5.2 | Source 15, Source 17, Source 19 | Note 2 |
|  |  |  |  |  | SU | 3.22 | 2.04~4.4 | Source 4, Source 17 | Note 2, 3 |
|  |  |  |  |  | MU | 7.07 | 5.3~8.4 | Source 7, Source 8, Source 9, Source 16, Source 18, Source 20 | Note 1 |
|  |  |  |  |  | MU | 2.4 | 2.4 | Source 11 | Note 2 |
|  |  |  |  | 120 | SU | 8.03 | 8.03 | Source 18 | Note 1 |
|  |  |  |  |  | MU | 11.42 | 11.42 | Source 18 | Note 1 |
|  |  |  | 30 Mbps | 30 | SU | 6.3 | 6.3 | Source 16 | Note 1 |
|  |  |  |  | 60 | SU | 7.31 | 6.54~8.4 | Source 3, Source 5, Source 7, Source 8, Source 9, Source 10, Source 14, Source 16, Source 18 | Note 1 |
|  |  |  |  |  | SU | 5.73 | 4.05~7.4 | Source 15, Source 17, Source 19 | Note 2 |
|  |  |  |  |  | SU | 6.23 | 4.05~8.4 | Source 4, Source 17 | Note 2,3 |
|  |  |  |  |  | MU | 11.41 | 7 ~ 13.59 | Source 6, Source 7, Source 8, Source 9, Source 10, Source 16, Source 18, Source 20 | Note 1 |
|  |  |  |  |  | MU | 3.9 | 3.9 | Source 11 | Note 2 |
|  |  |  |  |  | MU | 5.78 | 5.78 | Source 4 | Note 2, 3 |
|  |  |  |  | 120 | SU | 13.47 | 13.47 | Source 18 | Note 1 |
|  |  |  |  |  | MU | 20.78 | 20.78 | Source 18 | Note 1 |
|  |  | 7ms | 30 Mbps | 60 | MU | 7.35 | 6.3~ 8.4 | Source 8, Source 9 | Note 1 |
|  |  | 13ms | 30 Mbps | 60 | MU | 14.65 | 14.6~14.7 | Source 8, Source 9 | Note 1 |
|  | CG | 15 ms | 45 Mbps | 60 | SU | 6.3 | 6.3 | Source 17 | Note 2 |
|  |  |  |  |  | SU | 6.4 | 6.4 | Source 17 | Note2,3 |
|  |  |  | 30 Mbps | 60 | SU | 9.89 | 6.17~13 | Source 3, Source 5, Source 6, Source 7, Source 8, Source 9, Source 10, Source 14, Source 16, Source 18 | Note 1 |
|  |  |  |  |  | SU | 8.9 | 8~10.2 | Source 15, Source 17, Source 19 | Note 2 |
|  |  |  |  |  | SU | 7.94 | 5.57~10.3 | Source 4, Source 17 | Note 2, 3 |
|  |  |  |  |  | MU | 15.06 | 10.1~19.65 | Source 6, Source 7, Source 8, Source 9, Source 10, Source 16, Source 18, Source 20 | Note 1 |
|  |  |  |  |  | MU | 5 | 5 | Source 11 | Note 2 |
|  |  |  |  |  | MU | >8 | >8 | Source 4 | Note 2, 3 |
|  |  |  | 8 Mbps | 60 | SU |  | >20~>36 | Source 5, Source 7, Source 14, Source 16 | Note 1 |
|  |  |  |  |  | MU |  | >36~56.6 | Source 7, Source 16 | Note 1 |
| InH | AR/VR | 10 ms | 60Mbps | 60 | MU | 2 | 0~4 | Source 3, Source 16 |  |
|  |  |  | 45 Mbps | 60 | SU | 4.44 | 3.27~5 | Source 7, Source 14, Source 15, Source 16, Source 18, Source 19 |  |
|  |  |  |  |  | MU | 6.07 | 3.5~8 | Source 3, Source 7, Source 11, Source 16, Source 18, Source 20 |  |
|  |  |  |  | 120 | SU | 6.59 | 6.59 | Source 18 |  |
|  |  |  |  |  | MU | 9.22 | 9.22 | Source 18 |  |
|  |  |  | 30 Mbps | 60 | SU | 7.33 | 5.2~8.5 | Source 7, Source 14, Source 15, Source 16, Source 18, Source 19 |  |
|  |  |  |  |  | SU | 4.85 | 4.85 | Source 12 | Note3 |
|  |  |  |  |  | MU | 9.21 | 5~12 | Source 3, Source 6, Source 7, Source 11, Source 16, Source 18, Source 20 |  |
|  |  |  |  | 120 | SU | 11.63 | 11.63 | Source 18 |  |
|  |  |  |  |  | MU | 16.53 | 16.53 | Source 18 |  |
|  |  |  | 60 Mbps | 60 | MU | 4 | 4 | Source 3 |  |
|  |  | 7 ms | 30 Mbps | 60 | MU | 8 | 8 | Source 3 |  |
|  | CG | 15 ms | 30 Mbps | 60 | SU | 8.4 | 5.96~10.5 | Source 6, Source 7, Source 14, Source 15, Source 16, Source 18, Source 19 |  |
|  |  |  |  |  | SU | 9.4 | 9.4 | Source 12 | Note3 |
|  |  |  |  |  | MU | 11.96 | 7.2~16.2 | Source 3, Source 6, Source 7, Source 11, Source 16, Source 18, Source 20 |  |
|  |  |  | 8 Mbps | 60 | SU |  | >20~>38.7 | Source 7, Source 14, Source 16 |  |
|  |  |  |  |  | MU |  | >38.7~44.1 | Source 16 |  |
| UMa | AR/VR | 10 ms | 45 Mbps | 60 | SU | 3.62 | 1.8~4.7 | Source 5, Source 7, Source 8, Source 9, Source 14, Source 16, Source 18 | Note 1 |
|  |  |  |  |  | SU | 1.85 | 1.85 | Source 4 | Note 2, 3 |
|  |  |  |  |  | MU | 4.51 | 2.9~6 | Source 7, Source 8, Source 9, Source 16, Source 18, Source 20 | Note 1 |
|  |  |  |  | 120 | SU | 6.75 | 6.75 | Source 18 | Note 1 |
|  |  |  |  |  | MU | 8.12 | 8.12 | Source 18 | Note 1 |
|  |  |  | 30 Mbps | 60 | SU | 6.26 | 4.4~8 | Source 5, Source 7, Source 8, Source 9, Source 14, Source 16, Source 18 | Note 1 |
|  |  |  |  |  | SU | 2.98 | 2.98 | Source 4 | Note 2,3 |
|  |  |  |  |  | MU | 8.29 | 5.2~10 | Source 7, Source 8, Source 9, Source 16, Source 18, Source 20 | Note 1 |
|  |  |  |  | 120 | SU | 11.7 | 11.7 | Source 18 | Note 1 |
|  |  |  |  |  | MU | 14.59 | 14.59 | Source 18 | Note 1 |
|  | CG | 15 ms | 30 Mbps | 60 | SU | 8.36 | 5.4~10.33 | Source 5, Source 7, Source 8, Source 9, Source 14, Source 16, Source 18 | Note 1 |
|  |  |  |  |  | SU | 4.08 | 4.08 | Source 4 | Note 2,3 |
|  |  |  |  |  | MU | 11.59 | 8~14.33 | Source 7, Source 8, Source 9, Source 16, Source 18, Source 20 | Note 1 |
|  |  |  | 8 Mbps | 60 | SU |  | 17.5~32.9 | Source 5, Source 7, Source 14, Source 16 | Note 1 |
|  |  |  |  |  | MU |  | 23.8~>36 | Source 7, Source 16 | Note 1 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: BS antenna parameters: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1:8,2) Note 3: zero packet arrival interval among UEs |  |  |  |  |  |  |  |  |  |

Table 8.3.1.1-2. Summary of FR1 DL capacity evaluation results for multi-stream (I/P Frame Traffic Model)

| Scenario | Traffic model | App | R | α | [PER_I, PER_P] [PDB_I, PDB_P] | MIMO | Capacity result (UEs/cell) |  | Source | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | mean | range |  |  |
| DU | GOP-Based I/P Frame | VR/AR | 30 Mbps | 1 | [1%, 1%] [10ms, 10ms] | MU | 10 | 10 | Source 9 | Note 1 |
|  |  |  |  | 1.5 | [1%, 1%] [10ms, 10ms] | SU | 6.5 | 6.5 | Source 5 | Note 1 |
|  |  |  |  |  |  | MU | 7.62 | 6.74~8.5 | Source 9, Source 18 | Note 1 |
|  |  |  |  | 2 | [1%, 1%] [10ms, 10ms] | SU | 6.05 | 6~6.1 | Source 14, Source 5 | Note 1 |
|  |  |  |  |  | [1%, 1%] [10ms, 10ms] | MU | 7.57 | 5.2~10.8 | Source 9, Source 18, Source 20 | Note 1 |
|  |  |  |  | 3 | [1%, 1%] [10ms, 10ms] | MU | 3.11 | 2.21~4 | Source 9, Source 18 | Note 1 |
|  |  |  | 45 Mbps | 1.5 | [1%, 1%] [10ms, 10ms] | SU | 2 | 2 | Source 14 | Note 1 |
|  |  |  |  |  | [1%, 1%] [10ms, 10ms] | MU | 1.4 | 1.4 | Source 9 | Note 1 |
|  |  |  |  | 3 | [1%, 1%] [10ms, 10ms] | SU | - | <2 | Source 14 | Note 1 |
|  | Slice-Based I/P Frame | VR/AR | 30 Mbps | 1.5 | [1%, 1%] [10ms, 10ms] | MU | 13.78 | 13.78 | Source 18 | Note 1 |
|  |  |  |  | 2 | [1%, 1%] [10ms, 10ms] | MU | 13.76 | 12.7~14.9 | Source 9, Source 18, Source 20 | Note 1 |
|  |  |  |  | 3 | [1%, 1%] [10ms, 10ms] | MU | 13.77 | 13.77 | Source 18 | Note 1 |
| Uma | GOP-Based I/P Frame | VR/AR | 30 Mbps | 1.5 | [1%, 1%] [10ms, 10ms] | SU | 4.2 | 4.2 | Source 5 | Note 1 |
|  |  |  |  | 2 | [1%, 1%] [10ms, 10ms] | SU | 2.4 | 2.4 | Source 5 | Note 1 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |  |  |  |  |  |  |  |  |  |  |

Table 8.3.1.1-3. Summary of FR1 DL capacity evaluation results for multi-stream (Video stream 30Mbps+Data/audio stream 1.12Mbps)

| Scenario | App | PDB | R | MIMO | Capacity result (UEs/cell) |  | Source | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | mean | range |  |  |
| DU | VR/AR | 10ms for video stream; 30ms for audio stream | 30Mbps for video stream; 1.12Mbps for audio stream | SU | 6 | 6 | Source 1 | Note 1 |
| InH | VR/AR | 10ms for video stream; 30ms for audio stream | 30Mbps for video stream; 1.12Mbps for audio stream | SU | 4.1 | 4 | Source 1 |  |
| Note 1: BS antenna parameters: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1,8,2) |  |  |  |  |  |  |  |  |

##### DU Scenario

###### VR/AR

            - Single-stream traffic model

Based on the evaluation results in Table 8.3.1.1-1, the following observations can be made.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 64 TxRU BS antenna, it is observed from Source 3, Source 5, Source 7, Source 8, Source 9, Source 10, Source 14,  Source 16, Source 18 that mean capacity performance is 8.22 UEs per cell in a range of 5.1~10.6 UEs per cell.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 32 TxRU BS antenna, it is observed from Source 15, Source 17, Source 19 that mean capacity performance is 6.98 UEs per cell in a range of 6.54~7.4 UEs per cell.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with MU-MIMO and 64 TxRU BS antenna, it is observed from Source 6, Source 7, Source 8, Source 9, Source 10, Source 16, Source 18, Source 20 that mean capacity performance is 11.41 UEs per cell in a range of 7 ~ 13.59 UEs per cell.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with MU-MIMO and 32 TxRU BS antenna, it is observed from Source 11 that the capacity performance is 3.9 UEs per cell.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 45Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 64 TxRU BS antenna, it is observed from Source 5, Source 7, Source 8, Source 9,  Source 14, Source 16, Source 18 that the mean capacity performance is 4.58 UEs per cell in a range of 1.7~6 UEs per cell.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 45Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 32 TxRU BS antenna, it is observed from Source 15, Source 17, Source 19 that the mean capacity performance is 4.77 UEs per cell in a range of 4.1~5.2 UEs per cell.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 45Mbps, 10ms PDB, 60 FPS, with MU-MIMO and 64 TxRU BS antenna, it is observed from  Source 7, Source 8, Source 9, Source 16, Source 18, Source 20 that the mean capacity performance is 7.07 UEs per cell in a range of 5.3~8.4 UEs per cell.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 45Mbps, 10ms PDB, 60 FPS, with MU-MIMO and 32 TxRU BS antenna, it is observed from Source 11 that the mean capacity performance is 2.4 UEs per cell.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 60Mbps, 10ms PDB, 60 FPS, with 64 TxRU BS antenna and MU-MIMO, it is observed from Source 16 that the mean capacity performance is 0 UEs per cell.

            - Multi-stream traffic model

Based on the evaluation results in Table 8.3.1.1-2, the following observations can be made.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR Slice-Based I/P Frame Traffic Model, 30Mbps, [PDB_I, PDB_P] = [10ms, 10ms], [PER_I, PER_P] = [1%, 1%], with α = 1.5 and MU-MIMO, it is observed from Source 18 that the capacity performance is 13.78 UEs per cell.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR Slice-Based I/P Frame Traffic Model, 30Mbps, [PDB_I, PDB_P] = [10ms, 10ms], [PER_I, PER_P] = [1%, 1%], with α = 2 and MU-MIMO, it is observed from Source 9, Source 18, Source 20 that the mean capacity performance is 13.76 UEs per cell in a range of 12.7~14.9 UEs per cell.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR Slice-Based I/P Frame Traffic Model, 30Mbps, [PDB_I, PDB_P] = [10ms, 10ms], [PER_I, PER_P] = [1%, 1%], with α = 3 and MU-MIMO, it is observed from Source 18 that the capacity performance is 13.77 UEs per cell.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR GOP-Based I/P Frame Traffic Model, 30Mbps, [PDB_I, PDB_P] = [10ms, 10ms], [PER_I, PER_P] = [1%, 1%], with α = 1 and MU-MIMO, it is observed from Source 9 that the capacity performance is 10 UEs per cell.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR GOP-Based I/P Frame Traffic Model, 30Mbps, [PDB_I, PDB_P] = [10ms, 10ms], [PER_I, PER_P] = [1%, 1%], with α = 1.5 and SU-MIMO, it is observed from Source 5 that the capacity performance is 1.5 UEs per cell.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR GOP-Based I/P Frame Traffic Model, 30Mbps, [PDB_I, PDB_P] = [10ms, 10ms], [PER_I, PER_P] = [1%, 1%], with α = 1.5 and MU-MIMO, it is observed from Source 9, Source 18 that the mean capacity performance is 7.62 UEs per cell in a range of 6.74~8.5 UEs per cell.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR GOP-Based I/P Frame Traffic Model, 30Mbps, [PDB_I, PDB_P] = [10ms, 10ms], [PER_I, PER_P] = [1%, 1%], with α = 2 and SU-MIMO, it is observed from Source 5, Source 14 that the mean capacity performance is 6.05 UEs per cell in a range of 6~6.1 UEs per cell.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR GOP-Based I/P Frame Traffic Model, 30Mbps, [PDB_I, PDB_P] = [10ms, 10ms], [PER_I, PER_P] = [1%, 1%], with α = 2 and MU-MIMO, it is observed from Source 9, Source 18, Source 20 that the mean capacity performance is 7.57 UEs per cell in a range of 5.2~10.8 UEs per cell.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR GOP-Based I/P Frame Traffic Model, 30Mbps, [PDB_I, PDB_P] = [10ms, 10ms], [PER_I, PER_P] = [1%, 1%], with α = 3 and MU-MIMO, it is observed from Source 9, Source 18 that the mean capacity performance is 3.11 UEs per cell in a range of 2.21~4 UEs per cell.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR GOP-Based I/P Frame Traffic Model, 45Mbps, [PDB_I, PDB_P] = [10ms, 10ms], [PER_I, PER_P] = [1%, 1%], with α = 1.5 and MU-MIMO, it is observed from Source 9 that the capacity performance is 1.4 UEs per cell.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR GOP-Based I/P Frame Traffic Model, 45Mbps, [PDB_I, PDB_P] = [10ms, 10ms], [PER_I, PER_P] = [1%, 1%], with α = 2 and SU-MIMO, it is observed from Source 14 that the capacity performance is 2 UEs per cell.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR GOP-Based I/P Frame Traffic Model, 45Mbps, [PDB_I, PDB_P] = [10ms, 10ms], [PER_I, PER_P] = [1%, 1%], with α = 3 and SU-MIMO, it is observed from Source 14 that the capacity performance is <2 UEs per cell.

- For FR1, Dense Urban, DL with 100MHz bandwidth for VR/AR multi-stream traffic model with video stream 30Mbps+data/audio stream 1.12Mbps, [PDB_video, PDB_data/audio] = [10ms, 30ms], with SU-MIMO, it is observed from Source 1 that the capacity performance is 6 UEs/cell.

###### CG

Based on the evaluation results in Table 8.3.1.1-1, the following observations can be made.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for CG traffic model, 8Mbps, 15ms PDB, 60 FPS, with SU-MIMO and 64TxRU BS antenna, it is observed from  Source 5, Source 7, Source 14, Source 16 that the mean capacity performance is in a range of >20~>36 UEs per cell.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for CG traffic model, 8Mbps, 15ms PDB, 60 FPS, with MU-MIMO and 64TxRU BS antenna, it is observed from Source 7, Source 16 that the mean capacity performance is in a range of >36~56.6 UEs per cell.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for CG traffic model, 30Mbps, 15ms PDB, 60 FPS, with SU-MIMO and 64 TxRU BS antenna, it is observed from  Source 3, Source 5, Source 6, Source 7, Source 8, Source 9, Source 10, Source 14, Source 16, Source 18 that the mean capacity performance is 9.89 UEs per cell in a range of 6.17~13 UEs per cell.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for CG traffic model, 30Mbps, 15ms PDB, 60 FPS, with SU-MIMO and 32 TxRU BS antenna, it is observed from Source 15, Source 17, Source 19 that the mean capacity performance is 8.9 UEs per cell in a range of 8~10.2 UEs per cell.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for CG traffic model, 30Mbps, 15ms PDB, 60 FPS, with MU-MIMO and 64 TxRU BS antenna, it is observed from  Source 6, Source 7, Source 8, Source 9, Source 10, Source 16, Source 18, Source 20 that the mean capacity performance is 15.06 UEs per cell in a range of 10.1~19.65 UEs per cell.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for CG traffic model, 30Mbps, 15ms PDB, 60 FPS, with MU-MIMO and 64 TxRU BS antenna, with MU-MIMO and 32 TxRU BS antenna, it is observed from Source 11 that the mean capacity performance is 5 UEs per cell.

##### InH Scenario

###### VR/AR

            - Single stream traffic model

Based on the evaluation results in Table 8.3.1.1-1, the following observations can be made.

- For FR1, Indoor Hotspot, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO, it is observed from  Source 7, Source 14, Source 15, Source 16, Source 18, Source 19 that the mean capacity performance is 7.33 UEs per cell in a range of 5.2~8.5 UEs per cell.

- For FR1, Indoor Hotspot, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with MU-MIMO, it is observed from  Source 3, Source 6, Source 7, Source 11, Source 16, Source 18, Source 20 that the mean capacity performance is 9.21 UEs per cell in a range of 5~12 UEs per cell.

- For FR1, Indoor Hotspot, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 45Mbps, 10ms PDB, 60 FPS, with SU-MIMO, it is observed from  Source 7, Source 14, Source 15, Source 16, Source 18, Source 19 that the mean capacity performance is 4.44 UEs per cell in a range of 3.27~5 UEs per cell.

- For FR1, Indoor Hotspot, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 45Mbps, 10ms PDB, 60 FPS, with MU-MIMO, it is observed from  Source 3, Source 7, Source 11, Source 16, Source 18, Source 20 that the mean capacity performance is 6.07 UEs per cell in a range of 3.5~8 UEs per cell.

- For FR1, Indoor Hotspot, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 60Mbps, 10ms PDB, 60 FPS, with MU-MIMO, it is observed from Source 3, Source 16 that the mean capacity performance is 2 UEs per cell in a range of 0~4 UEs per cell.

            - Multi-stream traffic model

Based on the evaluation results in Table 8.3.1.1-3, the following observations can be made.

- For FR1, Indoor Hotspot, DL, with 100MHz bandwidth for VR/AR multi-stream traffic model with video stream 30Mbps+data/audio stream 1.12Mbps, PDB_video, PDB_data/audio = 10ms, 30ms, with SU-MIMO, it is observed from Source 1 that the capacity performance is 4.1.

###### CG

Based on the evaluation results in Table 8.3.1.1-1, the following observations can be made.

- For FR1, Indoor Hotspot, DL, with 100MHz bandwidth for CG traffic model, 8Mbps, 15ms PDB, 60 FPS, with SU-MIMO, it is observed from Source 7, Source 14, Source 16 that the capacity performance is in a range of >20~>38.7 UEs per cell.

- For FR1, Indoor Hotspot, DL, with 100MHz bandwidth for CG traffic model, 8Mbps, 15ms PDB, 60 FPS, with MU-MIMO, it is observed from Source 7, Source 16 that the capacity performance is in a range of >38.7~44.1 UEs per cell.

- For FR1, Indoor Hotspot, DL, with 100MHz bandwidth for CG traffic model, 30Mbps, with MU-MIMO, it is observed from Source 3, Source 6, Source 7, Source 11, Source 16, Source 18, Source 20 that the mean capacity performance is 11.96 UEs per cell in a range of 7.2~16.2 UEs per cell.

- For FR1, Indoor Hotspot, DL, with 100MHz bandwidth for CG traffic model, 30Mbps, with SU-MIMO, it is observed from Source 6, Source 7, Source 14, Source 15, Source 16, Source 18, Source 19 that the mean capacity performance is 8.4 UEs per cell in a range of 5.96~10.5 UEs per cell.

##### UMa Scenario

###### VR/AR

            - Single stream traffic model

Based on the evaluation results in Table 8.3.1.1-1, the following observations can be made.

- For FR1, Urban Macro, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 64 TxRU BS antenna, it is observed from  Source 5, Source 7, Source 8, Source 9, Source 14, Source 16, Source 18 that the mean capacity performance is 6.26 UEs per cell in a range of 4.4~8.

- For FR1, Urban Macro, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with MU-MIMO and 64 TxRU BS antenna, it is observed from Source 7, Source 8, Source 9, Source 16, Source 18, Source 20 that the mean capacity performance is 8.29 UEs per cell in a range of 5.2~10 UEs per cell.

- For FR1, Urban Macro, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 45Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 64 TxRU BS antenna, it is observed from  Source 5, Source 7, Source 8, Source 9, Source 14, Source 16, Source 18 that the mean capacity performance is 3.62 UEs per cell in a range of 1.8~4.7 UEs per cell.

- For FR1, Urban Macro, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 45Mbps, 10ms PDB, 60 FPS, with MU-MIMO and 64 TxRU BS antenna, it is observed from Source 7, Source 8, Source 9, Source 16, Source 18, Source 20 that the mean capacity performance is 4.51 UEs per cell in a range of 2.9~6 UEs per cell.

            - Multi-stream traffic model

Based on the evaluation results in Table 8.3.1.1-2, the following observations can be made.

- For FR1, Urban Macro, DL, with 100MHz bandwidth for VR/AR GOP-Based I/P Frame Traffic Model, 30Mbps, [PDB_I, PDB_P] = [10ms, 10ms], [PER_I, PER_P] = [1%, 1%], with α = 1.5 and SU-MIMO, it is observed from Source 5 that the capacity performance is 4.2 UEs per cell.

- For FR1, Urban Macro, DL, with 100MHz bandwidth for VR/AR GOP-Based I/P Frame Traffic Model, 30Mbps, [PDB_I, PDB_P] = [10ms, 10ms], [PER_I, PER_P] = [1%, 1%], with α = 2 and SU-MIMO, it is observed from Source 5 that the capacity performance is 2.4 UEs per cell.

###### CG

Based on the evaluation results in Table 8.3.1.1-1, the following observations can be made.

- For FR1, Urban Macro, DL, with 100MHz bandwidth for CG traffic model, 8Mbps, 15ms PDB, 60 FPS, with SU-MIMO and 64 TxRU BS antenna, it is observed from Source 5, Source 7, Source 14, Source 16 that the mean capacity performance is in a range of 17.5~32.9 UEs per cell.

- For FR1, Urban Macro, DL, with 100MHz bandwidth for CG traffic model, 8Mbps, 15ms PDB, 60 FPS, with MU-MIMO and 64 TxRU BS antenna, it is observed from Source 7, Source 16 that the mean capacity performance is in a range of 23.8~ >36 UEs per cell.

- For FR1, Urban Macro, DL, with 100MHz bandwidth for CG traffic model, 30Mbps, 15ms PDB, 60 FPS, with SU-MIMO and 64 TxRU BS antenna, it is observed from  Source 5, Source 7, Source 8, Source 9, Source 14, Source 16, Source 18 that the mean capacity performance is 8.36 UEs per cell in a range of 5.4~10.33 UEs per cell.

- For FR1, Urban Macro, DL, with 100MHz bandwidth for CG traffic model, 30Mbps, 15ms PDB, 60 FPS, with MU-MIMO, it is observed from  Source 7, Source 8, Source 9, Source 16, Source 18, Source 20 that the mean capacity performance is 11.59 UEs per cell in a range of 8~14.33 UEs per cell.

#### FR1 UL

This section captures the capacity baseline performance evaluation results of FR1 UL.

Table 8.3.1.2-1. Summary of UL capacity evaluation results in FR1

| Scenario | App | PDB (ms) | R | F(fps) | MIMO | Capacity result (UEs/cell) |  | Source | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | mean | range |  |  |
| DU | VR/CG (1 stream: Pose) | 10 | 0.2 Mbps | 250 | SU | - | 20 ~ 224.9 | Source 8, Source 14, Source 16, Source 18 | Note 1 |
|  |  |  |  |  | SU | 39.9 | 39.9 | Source 7 | Note 1,4 |
|  |  |  |  |  | SU | 45.77 | 45.77 | Source 15 | Note 2 |
|  |  |  |  |  | MU | - | >15 ~ >240 | Source 9, Source 16 | Note 1 |
|  | AR (1 stream: Scene) | 30 | 10 Mbps | 60 | SU | 7.80 | 4.5 ~ 9.49 | Source 10, Source 14, Source 16, Source 18 | Note 1 |
|  |  |  |  |  | SU | 7.45 | 7.4~7.5 | Source 7, Source 8 | Note 1,4 |
|  |  |  |  |  | SU | 4.77 | 4.77 | Source 15 | Note 2,3 |
|  |  |  |  |  | MU | 9.20 | 7.3~ 10.9 | Source 9, Source 10, Source 16, Source 20 | Note 1 |
|  |  |  |  |  | MU | 2.3 | 2.3 | Source 11 | Note 2,3 |
|  |  | 10 |  |  | MU | 0 | <1 | Source 9 | Note 1 |
|  |  | 15 |  |  | MU | 5.4 | 5.4 | Source 9 | Note 1 |
|  |  | 60 |  |  | MU | 8.3 | 8.3 | Source 9 | Note 1 |
|  |  | 30 | 20 Mbps | 60 | MU | 3.4 | 3.4 | Source 20 | Note 1 |
|  | AR (2 streams: Pose + Scene) | 10 (Pose), 30 (Scene) | 0.2 Mbps (Pose) 10 Mbps (Scene) | 250 (Pose) 60 (Scene) | SU | 4.37 | 2.6~ 7.43 | Source 7, Source 10, Source 16, Source 18 | Note 1 |
|  |  |  |  |  | MU | 3.96 | 1.5 ~ 5.8 | Source 9, Source 10, Source 16 | Note 1 |
|  |  |  |  |  | MU | 0 | 0 | Source 11 | Note 2 |
|  | AR (3 streams: Video +audio +Pose) | 10 (Pose), 30 (video), 10 (audio) | 0.2 Mbps (Pose) 10 Mbps (video) 1.12 Mbps (audio) | 250 (Pose) 60 (video) 100 (audio) | SU | 3.2 | 3 | Source 1 | Note 2 |
|  | AR (3 streams: Pose + I/P-stream) | 10 (Pose), 30 (I), 30 (P) | 0.2 Mbps (Pose) 10 Mbps (I+P) | 250 (Pose) 60 (I+P) | MU | 3.5 | 3.5 | Source 9 | Note 1 |
| InH | VR/CG (1 stream: Pose) | 10 | 0.2 Mbps | 250 | SU | - | 20 ~ 198 | Source 14, Source 15, Source 16, Source 18 |  |
|  |  |  |  |  | SU | - | >12~>40 | Source 3, Source 7 | Note4 |
|  |  |  |  |  | MU | - | >40~>240 | Source 16, Source 20 |  |
|  | AR (1 stream: Scene) | 30 | 10 Mbps | 60 | SU | 7.81 | 4.4 ~ 13.95 | Source 14, Source 16, Source 18 |  |
|  |  |  |  |  | SU | 4.66 | 4.66 | Source 15 | Note3 |
|  |  |  |  |  | SU | 6.05 | 6~6.1 | Source 3, Source 7 | Note4 |
|  |  |  |  |  | MU | 9.3 | 7.1 ~ 11.5 | Source 11, Source 16 |  |
|  | 2 streams: Pose + Scene | 10 (Pose), 30 (Scene) | 0.2 Mbps (Pose) 10 Mbps (Scene) | 250 (Pose) 60 (Scene) | SU | 8.41 | 4.1~ 12.71 | Source 16, Source 18 |  |
|  |  |  |  |  | SU | 5.8 | 5.8 | Source 7 | Note4 |
|  |  |  |  |  | MU | 7.3 | 7.2 ~ 7.4 | Source 11, Source 16 |  |
|  |  | 10 (Pose), 10 (Scene) | 0.2 Mbps (Pose) 10 Mbps (Scene) | 250 (Pose) 60 (Scene) | SU | 4.05 | 4.05 | Source 15 |  |
|  | 3 streams: Video + audio +Pose | 10 (Pose), 30 (video), 10 (audio) | 0.2 Mbps (Pose) 10 Mbps (video) 1.12 Mbps (audio) | 250 (Pose) 60 (video) 100 (audio) | SU | 4.1 | 4 | Source 1 |  |
| UMa | VR/CG (1 stream: Pose) | 10 | 0.2 Mbps | 250 | SU | - | 20 ~143 | Source 8, Source 14, Source 16, Source 18 | Note 1 |
|  |  |  |  |  | SU | 17.4 | 17.4 | Source 7 | Note 1,4 |
|  |  |  |  |  | MU | - | >15 ~ >240 | Source 9, Source 16 | Note 1 |
|  | AR (1 stream: Scene) | 30 | 10 Mbps | 60 | SU | - | 0 ~ 1.34 | Source 8, Source 14, Source 16, Source 18 | Note 1 |
|  |  |  |  |  | SU | - | <1 | Source 7 | Note 1,4 |
|  |  |  |  |  | MU | 0 | 0 ~ <1 | Source 9, Source 16 | Note 1 |
|  | AR (2 streams: pose + scene) | 10 (Pose), 30 (Scene) | 0.2 Mbps (Pose) 10 Mbps (Scene) | 250 (Pose) 60 (Scene) | SU | 0 | 0 | Source 16 | Note 1 |
|  |  |  |  |  | SU | - | <1 | Source 7 | Note 1,4 |
|  |  |  |  |  | MU | 0 | 0 | Source 16 | Note 1 |
|  |  |  |  |  | MU | - | <1 | Source 7 | Note 1,4 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: BS antenna parameters: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1:8,2) Note 3: With jitter Note 4: DDDUU |  |  |  |  |  |  |  |  |  |

##### DU Scenario

###### VR/CG (Pose/control-stream)

Based on the evaluation results in Table 8.3.1.2-1, the following observations can be made.

- For FR1, Dense Urban, UL, with 100MHz bandwidth for VR/CG Pose/control-stream, 0.2Mbps, 10ms PDB, 250 FPS, with SU-MIMO and 64 TxRU BS antenna, it is observed from Source 8, Source 14, Source 16, Source 18 that capacity performance is in a range of 20~224.9 UEs per cell.

- For FR1, Dense Urban, UL, with 100MHz bandwidth for VR/CG Pose/control-stream, 0.2Mbps, 10ms PDB, 250 FPS, with MU-MIMO and 64 TxRU BS antenna, it is observed from Source 9, Source 16 that capacity performance is in a range of >15~>240 UEs per cell.

- For FR1, Dense Urban, UL, with 100MHz bandwidth for VR/CG Pose/control-stream, 0.2Mbps, 10ms PDB, 250 FPS, with SU-MIMO and 32 TxRU BS antenna, it is observed from Source 15 that capacity performance is 45.77 UEs per cell.

- For FR1, Dense Urban, UL, with 100MHz bandwidth for VR/CG Pose/control-stream, 0.2Mbps, 10ms PDB, 250 FPS, with SU-MIMO, 64 TxRU BS antenna and DDDUU, it is observed from Source 7 that capacity performance is 39.9 UEs per cell.

###### AR (1 stream: Scene/video/data/voice-stream)

Based on the evaluation results in Table 8.3.1.2-1, the following observations can be made.

- For FR1, Dense Urban, UL, with 100MHz bandwidth for AR single-stream (Scene/video/data/ audio -stream, 10Mbps, 30ms PDB, 60FPS), with SU-MIMO and 64 TxRU BS antenna, it is observed from  Source 10, Source 14, Source 16,   Source 18 that the mean capacity performance is 7.80 UEs per cell in a range of 4.5~ 9.49 UEs per cell.

- For FR1, Dense Urban, UL, with 100MHz bandwidth for AR single-stream (Scene/video/data/audio-stream, 10Mbps, 30ms PDB, 60FPS), with MU-MIMO and 64 TxRU BS antenna, it is observed from Source 9, Source 10, Source 16, Source 20 that the mean capacity performance is 9.20 UEs per cell in a range of 7.3~10.9 UEs per cell.

- For FR1, Dense Urban, UL, with 100MHz bandwidth for AR single-stream (Scene/video/data/audio-stream, 10Mbps, 30ms PDB, 60FPS), with MU-MIMO, 64 TxRU BS antenna and DDDUU, it is observed from Source 7, Source 8 that the mean capacity performance is 7.45 UEs per cell in a range of 7.4~7.5 UEs per cell.

- For FR1, Dense Urban, UL, with 100MHz bandwidth for AR single-stream (Scene/video/data/audio-stream, 10Mbps, 30ms PDB, 60FPS), with SU-MIMO and 32 TxRU BS antenna, it is observed from Source 15 that the capacity performance is 4.77 UEs per cell.

- For FR1, Dense Urban, UL, with 100MHz bandwidth for AR single-stream (Scene/video/data/audio-stream, 10Mbps, 30ms PDB, 60FPS), with MU-MIMO and 32 TxRU BS antenna, it is observed from Source 11 that the capacity performance is 2.3 UEs per cell.

###### AR (2 streams: Pose/control-stream + scene/video/data/voice-stream)

Based on the evaluation results in Table 8.3.1.2-1, the following observations can be made.

- For FR1, Dense Urban, UL, with 100MHz bandwidth for AR two-stream (Scene/video/data/audio-stream, 10Mbps, 30ms PDB, 60FPS + Pose/control-stream, 0.2Mbps, 10ms PDB, 250 FPS), with SU-MIMO and 64 TxRU BS antenna, it is observed from Source 7, Source 10, Source 16, Source 18 that the mean capacity performance is 4.37 UEs per cell in a range of 2.6~7.43 UEs per cell.

- For FR1, Dense Urban, UL, with 100MHz bandwidth for AR two-stream (Scene/video/data/audio-stream, 10Mbps, 30ms PDB, 60FPS + Pose/control-stream, 0.2Mbps, 10ms PDB, 250 FPS), with MU-MIMO and 64 TxRU BS antenna, it is observed from  Source 9, Source 10, Source 16that the mean capacity performance is 3.96 UEs per cell in a range of 1.5~5.8 UEs per cell.

- For FR1, Dense Urban, UL, with 100MHz bandwidth for AR two-stream (Scene/video/data/audio-stream, 10Mbps, 30ms PDB, 60FPS + Pose/control-stream, 0.2Mbps, 10ms PDB, 250 FPS), with MU-MIMO and 32 TxRU BS antenna, it is observed from Source 11 that the capacity performance is 0 UEs per cell.

###### AR (3 streams: Video stream+Data/audio stream+Pose/control stream)

Based on the evaluation results in Table 8.3.1.2-1, the following observations can be made.

- For FR1, Dense Urban, UL, with 100MHz bandwidth for AR three-stream (Video-stream, 10Mbps, 30ms PDB, 60FPS + Audio/data-stream, 1.12Mbps, 10ms PDB, 100FPS + Pose/control-stream, 0.2Mbps, 10ms PDB, 250 FPS), with SU-MIMO and 64 32 TxRU BS antenna, it is identified observed from Source 1 that the capacity performance is 3 UEs per cell.

###### AR (3 streams: Pose/control-stream + I/P-stream)

Based on the evaluation results in Table 8.3.1.2-1, the following observations can be made.

- For FR1, Dense Urban, UL, with 100MHz bandwidth for AR three-stream (I/P-stream, 10Mbps, 30ms PDB, 60FPS + Pose/control-stream, 0.2Mbps, 10ms PDB, 250 FPS), with MU-MIMO and 64 TxRU BS antenna, it is observed from Source 9 that the capacity performance is 3.5 UEs per cell.

##### InH Scenario

###### VR/CG (Pose/control-stream)

Based on the evaluation results in Table 8.3.1.2-1, the following observations can be made.

- For FR1, Indoor Hotspot, UL, with 100MHz bandwidth for VR/CG Pose/control-stream, 0.2Mbps, 10ms PDB, 250 FPS, with SU-MIMO, it is observed from Source 14, Source 15, Source 16, Source 18 that capacity performance is in a range of 20~198 UEs per cell.

- For FR1, Indoor Hotspot, UL, with 100MHz bandwidth for VR/CG Pose/control-stream, 0.2Mbps, 10ms PDB, 250 FPS, with MU-MIMO, it is observed from Source 16, Source 20 that capacity performance is in a range of >40~>240 UEs per cell.

- For FR1, Indoor Hotspot, UL, with 100MHz bandwidth for VR/CG Pose/control-stream, 0.2Mbps, 10ms PDB, 250 FPS, with SU-MIMO and DDDUU, it is observed from Source 3, Source 7 that the capacity performance is in a range of >12~>40 UEs per cell.

###### AR (1 stream: Scene/video/data/voice-stream)

Based on the evaluation results in Table 8.3.1.2-1, the following observations can be made.

- For FR1, Indoor Hotspot, UL, with 100MHz bandwidth for AR single-stream (Scene/video/data/audio-stream, 10Mbps, 30ms PDB, 60FPS), with SU-MIMO, it is observed from Source 14, Source 16, Source 18 that the mean capacity performance is 7.81 UEs per cell in a range of 4.4~13.95 UEs per cell.

- For FR1, Indoor Hotspot, UL, with 100MHz bandwidth for AR single-stream (Scene/video/data/audio-stream, 10Mbps, 30ms PDB, 60FPS), with MU-MIMO, it is observed from Source 11, Source 16 that the mean capacity performance is 9.3 UEs per cell in a range of 7.1~11.5 UEs per cell.

- For FR1, Indoor Hotspot, UL, with 100MHz bandwidth for AR single-stream (Scene/video/data/audio-stream, 10Mbps, 30ms PDB, 60FPS), with SU-MIMO and DDDUU, it is observed from Source 3, Source 7 that the mean capacity performance is 6.05 UEs per cell in a range of 6~6.1 UEs per cell.

- For FR1, Indoor Hotspot, UL, with 100MHz bandwidth for AR single-stream (Scene/video/data/audio-stream, 10Mbps, 30ms PDB, 60FPS), with SU-MIMO and with jitter, it is observed from Source 15 that the capacity performance is 4.66 UEs per cell.

###### AR (2 streams: Pose/control-stream + scene/video/data/voice-stream)

Based on the evaluation results in Table 8.3.1.2-1, the following observations can be made.

- For FR1, Indoor Hotspot, UL, with 100MHz bandwidth for AR two-stream (Scene/video/data/audio-stream, 10Mbps, 30ms PDB, 60FPS + Pose/control-stream, 0.2Mbps, 10ms PDB, 250 FPS), with SU-MIMO, it is observed from Source 16, Source 18 that the mean capacity performance is 8.41 UEs per cell in a range of 4.1~12.71 UEs per cell.

- For FR1, Indoor Hotspot, UL, with 100MHz bandwidth for AR two-stream (Scene/video/data/audio-stream, 10Mbps, 10ms PDB, 60FPS + Pose/control-stream, 0.2Mbps, 10ms PDB, 250 FPS), with SU-MIMO, it is observed from Source 15, that the mean capacity performance is 4.05 UEs per cell.

- For FR1, Indoor Hotspot, UL, with 100MHz bandwidth for AR two-stream (Scene/video/data/audio-stream, 10Mbps, 30ms PDB, 60FPS + Pose/control-stream, 0.2Mbps, 10ms PDB, 250 FPS), with MU-MIMO, it is observed from Source 11, Source 16 that the mean capacity performance is 7.3 UEs per cell in a range of 7.2~7.4 UEs per cell.

- For FR1, Indoor Hotspot, UL, with 100MHz bandwidth for AR two-stream (Scene/video/data/audio-stream, 10Mbps, 30ms PDB, 60FPS + Pose/control-stream, 0.2Mbps, 10ms PDB, 250 FPS), with SU-MIMO and DDDUU, it is observed from Source 7 that the capacity performance is 5.8 UEs per cell.

###### AR (3 streams: Video stream+Data/audio stream+Pose/control stream)

Based on the evaluation results in Table 8.3.1.2-1, the following observations can be made.

- For FR1, Indoor Hotspot, UL, with 100MHz bandwidth for AR three-stream (Video-stream, 10Mbps, 30ms PDB, 60FPS + Audio/data-stream, 1.12Mbps, 10ms PDB, 100FPS + Pose/control-stream, 0.2Mbps, 10ms PDB, 250 FPS), with SU-MIMO and 32 TxRU BS antenna, it is observed from Source 1 that the capacity performance is 4 UEs per cell.

##### UMa Scenario

###### VR/CG (Pose/control-stream)

Based on the evaluation results in Table 8.3.1.2-1, the following observations can be made.

- For FR1, Urban Macro, UL, with 100MHz bandwidth for VR/CG Pose/control-stream, 0.2Mbps, 10ms PDB, 250 FPS, with SU-MIMO and 64 TxRU BS antenna, it is observed from Source 8, Source 14, Source 16, Source 18 that capacity performance is in a range of 20~143 UEs per cell.

- For FR1, Urban Macro, UL, with 100MHz bandwidth for VR/CG Pose/control-stream, 0.2Mbps, 10ms PDB, 250 FPS, with MU-MIMO and 64 TxRU BS antenna, it is observed from Source 9, Source 16 that capacity performance is in a range of >15~>240 UEs per cell.

- For FR1, Urban Macro, UL, with 100MHz bandwidth for VR/CG Pose/control-stream, 0.2Mbps, 10ms PDB, 250 FPS, with SU-MIMO, 64 TxRU BS antenna and DDDUU, it is observed from Source 7 that capacity performance is 17.4 UEs per cell.

###### AR (1 stream: Scene/video/data/voice-stream)

Based on the evaluation results in Table 8.3.1.2-1, the following observations can be made.

- For FR1, Urban Macro, UL, with 100MHz bandwidth for AR single-stream (Scene/video/data/audio-stream, 10Mbps, 30ms PDB, 60FPS), with SU-MIMO and 64 TxRU BS antenna, it is observed from Source 7, Source 8, Source 14, Source 16, Source 18 that the capacity performance is in a range of 0~1.34 UEs per cell.

- For FR1, Urban Macro, UL, with 100MHz bandwidth for AR single-stream (Scene/video/data/audio-stream, 10Mbps, 30ms PDB, 60FPS), with MU-MIMO and 64 TxRU BS antenna, it is observed from Source 9, Source 16 that the capacity performance is in a range of 0~<1 UE per cell.

###### AR (2 streams: Pose/control-stream + scene/video/data/voice-stream)

Based on the evaluation results in Table 8.3.1.2-1, the following observations can be made.

- For FR1, Urban Macro, UL, with 100MHz bandwidth for AR two-stream (Scene/video/data/audio-stream, 10Mbps, 30ms PDB, 60FPS + Pose/control-stream, 0.2Mbps, 10ms PDB, 250 FPS), with SU-MIMO and 64 TxRU BS antenna, it is observed from Source 7, Source 16 that the capacity performance is in a range of 0~<1 UE per cell.

- For FR1, Urban Macro, UL, with 100MHz bandwidth for AR two-stream (Scene/video/data/audio-stream, 10Mbps, 30ms PDB, 60FPS + Pose/control-stream, 0.2Mbps, 10ms PDB, 250 FPS), with MU-MIMO and 64 TxRU BS antenna, it is observed from Source 16 that the capacity performance is 0 UE per cell.

#### FR2 DL

This section captures the capacity baseline performance evaluation results of FR2 DL. This section captures the capacity baseline performance evaluation results of FR2 DL.

Table 8.3.1.3-1 Summary of FR2 DL capacity evaluation results for single stream (100MHz bandwidth)

| Scenario | App | PDB (ms) | R | F(fps) | MIMO | Capacity result (UEs/cell) |  | Source | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | mean | range |  |  |
| DU | AR/VR | 10 | 45 Mbps | 60 | SU | 5.71 | 3.94~8.2 | Source 15, Source 16, Source 18 | Note 1 |
|  |  |  |  |  | SU | 2.25 | 2~2.5 | Source 7, Source 16 | Note 1 3 |
|  |  |  |  |  | SU | 4.7 | 4.7 | Source 14 | Note 2 |
|  |  |  |  | 120 | SU | 7.91 | 5.5~10.32 | Source 16, Source 18 | Note 1 |
|  |  |  | 30 Mbps | 60 | SU | 8.93 | 6.35~13.44 | Source 15, Source 16, Source 18 | Note 1 |
|  |  |  |  |  | SU | 4.85 | 4.2~5.5 | Source 7, Source 16 | Note 1,3 |
|  |  |  |  |  | SU | 10 | 10 | Source 14 | Note 2 |
|  |  |  |  | 120 | SU | 11.64 | 7~16.28 | Source 16, Source 18 | Note 1 |
|  | CG | 15 | 30 Mbps | 60 | SU | 9.38 | 5.1~16.16 | Source 7, Source 15, Source 16, Source 18 | Note 1 |
|  |  |  |  |  | SU | 11 | 11 | Source 14 | Note 2 |
|  |  |  | 8Mbps | 60 | SU | 32.5 | 32.5 | Source 16 | Note 1 |
|  |  |  |  |  | SU | >20 | >20 | Source 14 | Note 2 |
| InH | AR/VR | 10 | 45 Mbps | 60 | SU | 4.74 | 3.2~6.09 | Source 7, Source 15, Source 16, Source 18 | Note 1 |
|  |  |  |  |  | SU | 2.5 | 2.5 | Source 16 | Note 1, 3 |
|  |  |  |  |  | SU | 4.7 | 4.7 | Source 14 | Note 2 |
|  |  |  |  | 120 | SU | 5.77 | 5.5~6.03 | Source 16, Source 18 | Note 1 |
|  |  |  | 30 Mbps | 60 | SU | 8.02 | 6.2~10.17 | Source 7, Source 15, Source 16, Source 18 | Note 1 |
|  |  |  |  |  | SU | 5.5 | 5.5 | Source 16 | Note 1, 3 |
|  |  |  |  |  | SU | 8.9 | 7.8~ 10 | Source 14, Source 20 | Note 2 |
|  |  |  |  |  | SU | 7.8 | 7.8 | Source 20, | Note 2, 4 |
|  |  |  |  | 120 | SU | 8.87 | 7.5~10.23 | Source 16, Source 18 |  |
|  | CG | 15 | 30 Mbps | 60 | SU | 8.94 | 6.9~11.45 | Source 7, Source 15, Source 16, Source 18 | Note 1 |
|  |  |  |  |  | SU | 10.45 | 9.9~ 11 | Source 14, Source 20 | Note 2 |
|  |  |  |  |  | SU | 9.9 | 9.9 | Source 20 | Note 2, 4 |
|  |  |  | 8 Mbps | 60 | SU | 29.5 | 28~31 | Source 7, Source 16 | Note 1 |
|  |  |  |  |  | SU | >20 | >20 | Source 14 | Note 2 |
| Note 1: UE Antenna parameters: Option 1: (M, N, P)=(1, 4, 2), 3 panels (left, right, top) Note 2: UE Antenna parameters: Option 2: 4Tx/4Rx: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2), (dH,dV) = (0.5, 0.5)λ Note 3: DDDUU Note 4: 64 QAM |  |  |  |  |  |  |  |  |  |

Table 8.3.1.3-2. Summary of FR2 DL capacity evaluation results for single stream (400MHz bandwidth)

| Scenario | App | PDB (ms) | R | F(fps) | MIMO | Capacity result (UEs/cell) |  | Source | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | mean | data |  |  |
| DU | AR/VR | 10 | 45 Mbps | 60 | SU | 33.20 | 22.5~43.89 | Source 16, Source 18 | Note 2 |
|  |  |  |  |  | SU | 16.5 | 16.5 | Source 16 | Note 1, 2 |
|  |  |  | 30 Mbps | 60 | SU | 30 | 30 | Source 16 | Note 2 |
|  |  |  |  |  | SU | 21.5 | 21.5 | Source 16 | Note 1, 2 |
|  | CG | 15 | 30 Mbps | 60 | SU | 32.5 | 32.5 | Source 16 | Note 2 |
|  |  |  | 8 Mbps | 60 | SU | >45 | >45 | Source 16 | Note 2 |
| InH | AR/VR | 10 | 45 Mbps | 60 | SU | 19 | 19 | Source 16 | Note 1, 2 |
|  |  |  |  |  | SU | 27 | 27 | Source 16 | Note 2 |
|  |  |  | 30 Mbps | 60 | SU | 34 | 34 | Source 16 | Note 2 |
|  |  |  |  |  | SU | 25 | 25 | Source 16 | Note 1, 2 |
|  | CG | 15 | 30 Mbps | 60 | SU | 36 | 36 | Source 16 | Note 2 |
|  |  |  | 8 Mbps | 60 | SU | 44 | 44 | Source 16 | Note 2 |
| Note 1: DDDDU Note 2: UE Antenna parameters: Option 1: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) |  |  |  |  |  |  |  |  |  |

Table 8.3.1.3-3. Summary of FR2 DL capacity evaluation results for multi stream (Video + Audio/data)

| Scenario | R of video-stream | Video PDB (ms) | R of audio-stream | Audio PDB (ms) | MIMO | Capacity result (UEs/cell) |  | Source | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | mean | data |  |  |
| DU | 30 Mbps | 10 | 0.756 Mbps | 30 | SU | 6 | 6 | Source 16 | Note1 |
|  |  |  |  |  | SU | 3.5 | 3.5 | Source 16 | Note1,2 |
| InH | 30 Mbps | 10 | 0.756 Mbps | 30 | SU | 6 | 6 | Source 16 | Note1 |
|  |  |  |  |  | SU | 4 | 4 | Source 16 | Note1,2 |
| Note 2: UE antenna configuraiton: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) Note 3: DDDUU |  |  |  |  |  |  |  |  |  |

Table 8.3.1.3-4. Summary of FR2 DL capacity evaluation results for multi-stream (I/P Frame Traffic Model)

| Scenario | Traffic model | App | R | α | MIMO | Capacity result (UEs/cell) |  | Source | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | mean | data |  |  |
| InH | GOP-Based I/P Frame | VR/AR | 30 Mbps | 1.5 | SU | 5.37 | 5.37 | Source 18 | Note 1 |
|  |  |  |  | 2 | SU | 3.53 | 3.53 | Source 18 | Note 1 |
|  |  |  |  | 3 | SU | 2.29 | 2.29 | Source 18 | Note 1 |
|  | Slice-Based I/P Frame | VR/AR | 30 Mbps | 1.5 | SU | 8.23 | 8.23 | Source 18 | Note 1 |
|  |  |  |  | 2 | SU | 8.24 | 8.24 | Source 18 | Note 1 |
|  |  |  |  | 3 | SU | 8.23 | 8.23 | Source 18 | Note 1 |
| Note 1: [PDB_I, PDB_P] = [10, 10]; [PER_I, PER_P] = [1%, 1%] |  |  |  |  |  |  |  |  |  |

##### DU Scenario

###### VR/AR

            - Single stream traffic model

Based on the evaluation results in Table 8.3.1.3-1 and Table 8.3.1.3-2, the following observations can be made.

- For FR2, Dense Urban DL, with 100MHz bandwidth for VR/AR single-stream traffic model, with SU-MIMO and Option 1 UE Antenna parameters: (M, N, P) = (1, 4, 2), 3 panels (left, right, top), 30Mbps, 10ms PDB, 60 FPS, it is observed from Source 15, Source 16, Source 18 that mean capacity performance is 8.93 UEs per cell in a range of 6.35~13.44 UEs per cell.

- For FR2, Dense Urban DL, with 100MHz bandwidth for VR/AR single-stream traffic model, with SU-MIMO and Option 1 UE Antenna parameters: (M, N, P) = (1, 4, 2), 3 panels (left, right, top), 30Mbps, DDDUU, 10ms PDB, 60 FPS, it is observed from Source 15, Source 16, Source 18 that mean capacity performance is 4.85 UEs per cell in a range of 4.2~5.5 UEs per cell.

- For FR2, Dense Urban DL, with 100MHz bandwidth for VR/AR single-stream traffic model, with SU-MIMO and Option 1 UE Antenna parameters: (M, N, P) = (1, 4, 2), 3 panels (left, right, top), 45Mbps, 10ms PDB, 60 FPS, it is observed from Source 15, Source 16, Source 18 that mean capacity performance is 5.71 UEs per cell in a range of 3.94~8.2 UEs per cell.

- For FR2, Dense Urban DL, with 100MHz bandwidth for VR/AR single-stream traffic model, with SU-MIMO and Option 1 UE Antenna parameters: (M, N, P) = (1, 4, 2), 3 panels (left, right, top), 45Mbps, DDDUU, 10ms PDB, 60 FPS, it is observed from Source 7, Source 16 that mean capacity performance is 2.25 UEs per cell in a range of 2~2.5 UEs per cell.

- For FR2, Dense Urban, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, with SU-MIMO, 10ms PDB, 60 FPS, 30Mbps, Option 2 UE Antenna parameters: 4Tx/4Rx: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2), (dH,dV) = (0.5, 0.5)λ, it is observed from Source 14 that the capacity performance is 10 UEs per cell.

- For FR2, Dense Urban, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, with SU-MIMO, 10ms PDB, 120 FPS, 30Mbps, Option 1 UE Antenna parameters: (M, N, P) = (1, 4, 2), 3 panels (left, right, top), it is observed from Source 18, Source 16 that the mean capacity performance is 11.64 UEs per cell in a range of 7~16.28.

- For FR2, Dense Urban, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, with SU-MIMO, 10ms PDB, 60 FPS, 45Mbps, Option 2 UE Antenna parameters: 4Tx/4Rx: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2), (dH,dV) = (0.5, 0.5)λ, it is observed from Source 14 that the capacity performance is 4.7 UEs per cell.

- For FR2, Dense Urban, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, with SU-MIMO, 10ms PDB, 120 FPS, 45Mbps, Option 1 UE Antenna parameters: (M, N, P) = (1, 4, 2), 3 panels (left, right, top), it is observed from Source 18, Source 16 that the mean capacity performance is 7.91 UEs per cell in a range of 5.5~10.32 UEs per cell.

- For FR2, Dense Urban, DL, with 400MHz bandwidth for VR/AR single-stream traffic model, with SU-MIMO, 10ms PDB, 60 FPS, 30Mbps, Option 1 UE Antenna parameters: (M, N, P) = (1, 4, 2), 3 panels (left, right, top), it is observed from Source 16 that the capacity performance is 30 UEs per cell.

- For FR2, Dense Urban, DL, with 400MHz bandwidth for VR/AR single-stream traffic model, with SU-MIMO, 10ms PDB, 60 FPS, 30Mbps, DDDDU, Option 1 UE Antenna parameters: (M, N, P) = (1, 4, 2), 3 panels (left, right, top), it is observed from Source 16 that the capacity performance is 21.5 UEs per cell.

- For FR2, Dense Urban, DL, with 400MHz bandwidth for VR/AR single-stream traffic model, with SU-MIMO, 10ms PDB, 60 FPS, 45Mbps, Option 1 UE Antenna parameters: (M, N, P) = (1, 4, 2), 3 panels (left, right, top), it is observed from Source 16, Source 18 that mean capacity performance is 33.20 UEs per cell in a range of 22.5~43.89 UEs per cell.

- For FR2, Dense Urban, DL, with 400MHz bandwidth for VR/AR single-stream traffic model, with SU-MIMO, 10ms PDB, 60 FPS, 45Mbps and DDDDU, Option 1 UE Antenna parameters: (M, N, P) = (1, 4, 2), 3 panels (left, right, top), it is observed from Source 16 that the capacity performance is 16.5 UEs per cell.

            - Multi-stream traffic model

Based on the evaluation results in Table 8.3.1.3-3, the following observations can be made.

- For FR2, Dense Urban, DL, with 100MHz bandwidth for Video + Audio/data multi-stream traffic model, with SU-MIMO, 10ms Video PDB,30ms Audio PDB, Option 1 UE Antenna parameters: (M, N, P) = (1, 4, 2), 3 panels (left, right, top), it is observed from Source 16 that the capacity performance is 6 UEs per cell.

- For FR2, Dense Urban, DL, with 100MHz bandwidth for Video + Audio/data multi-stream traffic model, with SU-MIMO, 10ms Video PDB,30ms Audio PDB and DDDUU, it is observed from Source 16 that the capacity performance is 3.5 UEs per cell.

###### CG

Based on the evaluation results in Table 8.3.1.3-1 and Table 8.3.1.3-2, the following observations can be made.

- For FR2, Dense Urban DL, with 100MHz bandwidth for CG single-stream traffic model, with SU-MIMO and Option 1 UE Antenna parameters: (M, N, P) = (1, 4, 2), 3 panels (left, right, top), 30Mbps, 15ms PDB, 60 FPS, it is observed from Source 7, Source 15, Source 16, Source 18 that mean capacity performance is 9.38 UEs per cell in a range of 5.1~16.16 UEs per cell.

- For FR2, Dense Urban, DL, with 100MHz bandwidth for CG single-stream traffic model, with SU-MIMO, 15ms PDB, 60 FPS, 30Mbps, Option 2 UE Antenna parameters: 4Tx/4Rx: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2), (dH,dV) = (0.5, 0.5)λ, DDDSU, it is observed from Source 16 that the capacity performance is 11 UEs per cell.

- For FR2, Dense Urban, DL, with 100MHz bandwidth for CG single-stream traffic model, with SU-MIMO, 15ms PDB, 60 FPS, 8Mbps, Option 1 UE Antenna parameters: (M, N, P) = (1, 4, 2), 3 panels (left, right, top), it is observed from Source 16 that the capacity performance is 32.5 UEs per cell.

- For FR2, Dense Urban, DL, with 100MHz bandwidth for CG single-stream traffic model, with SU-MIMO, 15ms PDB, 60 FPS, 8Mbps, Option 2 UE Antenna parameters: 4Tx/4Rx: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2), (dH,dV) = (0.5, 0.5)λ, it is observed from Source 14 that the capacity performance is > 20 UEs per cell.

- For FR2, Dense Urban, DL, with 400MHz bandwidth for CG single-stream traffic model, with SU-MIMO, 15ms PDB, 60 FPS, 30Mbps and Option 1 UE Antenna parameters: (M, N, P) = (1, 4, 2), 3 panels (left, right, top), it is observed from Source 14 that the capacity performance is 32.5 UEs per cell.

- For FR2, Dense Urban, DL, with 400MHz bandwidth for CG single-stream traffic model, with SU-MIMO, 15ms PDB, 60 FPS, 8Mbps, Option 1 UE Antenna parameters: (M, N, P) = (1, 4, 2), 3 panels (left, right, top), it is observed from Source 16 that the capacity performance is > 45 UEs per cell.

##### InH Scenario

###### VR/AR

            - Single-stream traffic model

Based on the evaluation results in Table 8.3.1.3-1 and Table 8.3.1.3-2, the following observations can be made.

- For FR2, Indoor Hotspot DL, with 100MHz bandwidth for VR/AR single-stream traffic model, with SU-MIMO and Option 1 UE Antenna parameters: (M, N, P) = (1, 4, 2), 3 panels (left, right, top), 45Mbps, 10ms PDB, 60 FPS, it is observed from Source 7, Source 15, Source 16, Source 18 that mean capacity performance is 4.74 UEs per cell in a range of 3.2~6.09 UEs per cell.

- For FR2, Indoor Hotspot DL, with 100MHz bandwidth for VR/AR single-stream traffic model, with SU-MIMO and Option 1 UE Antenna parameters: (M, N, P) = (1, 4, 2), 3 panels (left, right, top), 30Mbps, 10ms PDB, 60 FPS, it is observed from  Source 7, Source 15, Source 16, Source 18 that mean capacity performance is 8.02 UEs per cell in a range of 6.2~10.17 UEs per cell.

- For FR2, Indoor Hotspot DL, with 100MHz bandwidth for VR/AR single-stream traffic model, with SU-MIMO and Option 2 UE Antenna parameters: 4Tx/4Rx: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2), (dH,dV) = (0.5, 0.5)λ, 30Mbps, 10ms PDB, 60 FPS, it is observed from Source 14, Source 20 that mean capacity performance is 8.9 UEs per cell in a range of 7.8~10 UEs per cell.

- For FR2, Indoor Hotspot DL, with 100MHz bandwidth for VR/AR single-stream traffic model, with SU-MIMO, 10ms PDB, 60 FPS, 30Mbps, DDDUU and Option 1 UE Antenna parameters: (M, N, P) = (1, 4, 2), 3 panels (left, right, top), it is observed from Source 16 that the capacity performance is 5.5 UEs per cell.

- For FR2, Indoor Hotspot DL, with 100MHz bandwidth for VR/AR single-stream traffic model, with SU-MIMO, 10ms PDB, 60 FPS, 30Mbps, Option 2 UE Antenna parameters: 4Tx/4Rx: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2), (dH,dV) = (0.5, 0.5)λ, it is observed from Source 20 that the capacity performance is 7.8 UEs per cell.

- For FR2, Indoor Hotspot DL, with 100MHz bandwidth for VR/AR single-stream traffic model, with SU-MIMO, 10ms PDB, 120 FPS, 30Mbps, Option 1 UE Antenna parameters: (M, N, P) = (1, 4, 2), 3 panels (left, right, top), it is observed from Source 16, Source 18 that the mean capacity performance is 8.87 UEs per cell in a range of 7~10.23 UEs per cell.

- For FR2, Indoor Hotspot DL, with 100MHz bandwidth for VR/AR single-stream traffic model, with SU-MIMO, 10ms PDB, 60 FPS, 45Mbps, DDDUU, Option 1 UE Antenna parameters: (M, N, P) = (1, 4, 2), 3 panels (left, right, top), it is observed from Source 16 that the capacity performance is 2.5 UEs per cell.

- For FR2, Indoor Hotspot DL, with 100MHz bandwidth for VR/AR single-stream traffic model, with SU-MIMO, 10ms PDB, 60 FPS, 45Mbps, Option 2 UE Antenna parameters: 4Tx/4Rx: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2), (dH,dV) = (0.5, 0.5)λ, it is observed from Source 14 that the capacity performance is 4.7 UEs per cell.

- For FR2, Indoor Hotspot DL, with 100MHz bandwidth for VR/AR single-stream traffic model, with SU-MIMO, 10ms PDB, 120 FPS, 45Mbps, Option 1 UE Antenna parameters: (M, N, P) = (1, 4, 2), it is observed from Source 16, Source 18 that the mean capacity performance is 5.77 UEs per cell in a range of 5.5~6.03 UEs per cell.

- For FR2, Indoor Hotspot DL, with 400MHz bandwidth for VR/AR single-stream traffic model, with SU-MIMO, 10ms PDB, 60 FPS, 30Mbps, DDDUU, Option 1 UE Antenna parameters: (M, N, P) = (1, 4, 2), 3 panels (left, right, top), it is observed from Source 16 that the capacity performance is 25 UEs per cell.

- For FR2, Indoor Hotspot DL, with 400MHz bandwidth for VR/AR single-stream traffic model, with SU-MIMO, 10ms PDB, 60 FPS, 30Mbps, Option 1 UE Antenna parameters: (M, N, P) = (1, 4, 2), 3 panels (left, right, top), it is observed from Source 16 that the capacity performance is 34 UEs per cell.

- For FR2, Indoor Hotspot DL, with 400MHz bandwidth for VR/AR single-stream traffic model, with SU-MIMO, 10ms PDB, 60 FPS, 45Mbps, Option 1 UE Antenna parameters: (M, N, P) = (1, 4, 2), 3 panels (left, right, top), it is observed from Source 16 that the capacity performance is 27 UEs per cell.

- For FR2, Indoor Hotspot DL, with 400MHz bandwidth for VR/AR single-stream traffic model, with SU-MIMO, 10ms PDB, 60 FPS, 45Mbps, DDDDU, Option 1 UE Antenna parameters: (M, N, P) = (1, 4, 2), 3 panels (left, right, top), it is observed from Source 16 that the capacity performance is 19 UEs per cell.

            - Multi-stream traffic model

Based on the evaluation results in Table 8.3.1.3-4, the following observations can be made.

- For FR2, Indoor Hotspot DL, with 100MHz bandwidth for Video+Audio/data multi-stream Traffic Model, with SU-MIMO, 10ms Video PDB, 30ms Audio PDB, Option 1 UE Antenna parameters: (M, N, P) = (1, 4, 2), 3 panels (left, right, top), it is observed from Source 16 that the capacity performance is 6 UEs per cell.

- For FR2, Indoor Hotspot DL, with 100MHz bandwidth for Video+Audio/data multi-stream Traffic Model, with SU-MIMO, 10ms Video PDB, 30ms Audio PDB and DDDUU TDD format, it is observed from Source 16 that the capacity performance is 4 UEs per cell.

- For FR2, Indoor Hotspot DL, with 100MHz bandwidth for GOP-Based I/P Frame Traffic Model, with SU-MIMO,30Mbps, [PDB_I, PDB_P] = [10ms, 10ms], [PER_I, PER_P] = [1%, 1%], it is observed from Source 18 that the capacity performance is 5.37 UEs per cell with α = 1.5.

- For FR2, Indoor Hotspot DL, with 100MHz bandwidth for GOP-Based I/P Frame Traffic Model, with SU-MIMO,30Mbps, [PDB_I, PDB_P] = [10ms, 10ms], [PER_I, PER_P] = [1%, 1%], it is observed from Source 18 that the capacity performance is 3.53 UEs per cell with α = 2.

- For FR2, Indoor Hotspot DL, with 100MHz bandwidth for GOP-Based I/P Frame Traffic Model, with SU-MIMO,30Mbps, [PDB_I, PDB_P] = [10ms, 10ms], [PER_I, PER_P] = [1%, 1%], it is observed from Source 18 that the capacity performance is 2.29 UEs per cell with α = 3.

- For FR2, Indoor Hotspot DL, with 100MHz bandwidth for Slice-Based I/P Frame Traffic Model, with SU-MIMO,30Mbps, [PDB_I, PDB_P] = [10ms, 10ms], [PER_I, PER_P] = [1%, 1%], it is observed from Source 18 that the capacity performance is 8.23 UEs per cell with α = 1.5.

- For FR2, Indoor Hotspot DL, with 100MHz bandwidth for Slice-Based I/P Frame Traffic Model, with SU-MIMO,30Mbps, [PDB_I, PDB_P] = [10ms, 10ms], [PER_I, PER_P] = [1%, 1%], it is observed from Source 18 that the capacity performance is 8.24 UEs per cell with α = 2.

- For FR2, Indoor Hotspot DL, with 100MHz bandwidth for Slice-Based I/P Frame Traffic Model, with SU-MIMO,30Mbps, [PDB_I, PDB_P] = [10ms, 10ms], [PER_I, PER_P] = [1%, 1%], it is observed from Source 18 that the capacity performance is 8.23 UEs per cell with α = 3.

###### CG

Based on the evaluation results in Table 8.3.1.3-1 and Table 8.3.1.3-2, the following observations can be made.

- For FR2, Indoor Hotspot DL, with 100MHz bandwidth for CG single-stream traffic model, with SU-MIMO and Option 1 UE Antenna parameters: (M, N, P) = (1, 4, 2), 3 panels (left, right, top), 30Mbps, 15ms PDB, 60 FPS, it is observed from  Source 7, Source 15, Source 16, Source 18 that mean capacity performance is 8.94 UEs per cell in a range of 6.9~11.45 UEs per cell.

- For FR2, Indoor Hotspot DL, with 100MHz bandwidth for CG single-stream traffic model, with SU-MIMO and Option 2 UE Antenna parameters: 4Tx/4Rx: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2), (dH,dV) = (0.5, 0.5)λ, 30Mbps, 15ms PDB, 60 FPS, it is observed from  Source 14, Source 20 that mean capacity performance is 10.45 UEs per cell in a range of 9.9~11 UEs per cell.

- For FR2, Indoor Hotspot DL, with 100MHz bandwidth for CG single-stream traffic model, with SU-MIMO and Option 1 UE Antenna parameters: (M, N, P) = (1, 4, 2), 3 panels (left, right, top), 8Mbps, 15ms PDB, 60 FPS, it is observed from Source 7, Source 16 that mean capacity performance is 29.5 UEs per cell in a range of 28~31 UEs per cell.

- For FR2, Indoor Hotspot DL, with 100MHz bandwidth for CG single-stream traffic model, with SU-MIMO, 15ms PDB, 60 FPS, 30Mbps, Option 2 UE Antenna parameters: 4Tx/4Rx: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2), (dH,dV) = (0.5, 0.5)λ, it is observed from Source 20 that the capacity performance is 9.9 UEs per cell.

- For FR2, Indoor Hotspot DL, with 100MHz bandwidth for CG single-stream traffic model, with SU-MIMO, 15ms PDB, 60 FPS, 8 Mbps, Option 2 UE Antenna parameters: 4Tx/4Rx: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2), (dH,dV) = (0.5, 0.5)λ, it is observed from Source 14 that the capacity performance is > 20 UEs per cell.

- For FR2, Indoor Hotspot DL, with 400MHz bandwidth for CG single-stream traffic model, with SU-MIMO, 15ms PDB, 60 FPS, 30 Mbps, Option 1 UE Antenna parameters: (M, N, P) = (1, 4, 2), 3 panels (left, right, top), it is observed from Source 16 that the capacity performance is 36 UEs per cell.

- For FR2, Indoor Hotspot DL, with 400MHz bandwidth for CG single-stream traffic model, with SU-MIMO, 15ms PDB, 60 FPS, 8 Mbps, Option 1 UE Antenna parameters: (M, N, P) = (1, 4, 2), 3 panels (left, right, top), it is observed from Source 16 that the capacity performance is 44 UEs per cell.

#### FR2 UL

This section captures the capacity baseline performance evaluation results of FR2 UL.

Table 8.3.1.4-1. Summary of UL capacity evaluation results in FR2

| Scenario | App | PDB (ms) | R (Mbps) |  | F(fps) | MIMO | Capacity result (UEs/cell) |  | Source | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | mean | range |  |  |
| DU | VR/CG (Pose/control-stream) | 10 | 0.2 | 250 |  | SU | 20 | 20 | Source 18 | Note 1 |
|  |  |  |  |  |  | SU | 7.5 | 7.5 | Source 16 | Note 1,2,3 |
|  |  |  |  |  |  | SU | 18.5 | 18.5 | Source 16 | Note 1,2,3,6 |
|  |  |  |  |  |  | SU | >30 | >30 | Source 14 | Note 5 |
|  | AR (1 stream: Scene/video/data/voice-stream) | 30 | 10 | 60 |  | SU | 8.3 | 8.3 | Source 18 | Note 1 |
|  |  |  |  |  |  | SU | 1.29 | 1.29 | Source 14 | Note 5 |
|  |  |  |  |  |  | SU | 5 | 5 | Source 16 | Note 1 |
|  |  |  |  |  |  | SU | 9 | 9 | Source 16 | Note 1,6 |
|  |  | 15 | 20 |  |  | SU | 3.5 | 3.5 | Source 16 | Note 1,6 |
|  |  | 30 |  |  |  | SU | 5 | 5 | Source 16 | Note 1,6 |
|  |  | 60 |  |  |  | SU | 5 | 5 | Source 16 | Note 1,6 |
|  | AR (2 streams: Pose/control-stream + scene/video/data/voice-stream) | 10 (Pose), 30 (Scene) | 0.2 (Pose) 10 (Scene) | 250 (Pose) 60 (Scene) |  | SU | 1.5 | 1.5 | Source 16 | Note 1,6 |
|  |  |  |  |  |  | SU | 4.5 | 4.5 | Source 16 | Note 1 |
|  |  |  | 0.2 (Pose) 20 (Scene) |  |  | SU | 2 | 2 | Source 16 | Note 1,6 |
| InH | VR/CG (Pose/control-stream) | 10 | 0.2 | 250 |  | SU | 20 | 20 | Source 18 | Note 1 |
|  |  |  |  |  |  | SU | 7 | 7 | Source 16 | Note 1,2,3 |
|  |  |  |  |  |  | SU | 19 | 19 | Source 16 | Note 1,2,3,6 |
|  |  |  |  |  |  | SU | 12.09 | 12.09 | Source 14 | Note 5 |
|  | AR (1 stream: Scene/video/data/voice-stream) | 30 | 10 | 60 |  | SU | 8.59 | 8.59 | Source 18 | Note 1 |
|  |  |  |  |  |  |  | 1 | 1 | Source 14 | Note 5 |
|  |  |  |  |  |  |  | 5 | 5 | Source 16 | Note 1 |
|  |  |  |  |  |  |  | 10 | 10 | Source 16 | Note 1,6 |
|  |  | 15 | 20 |  |  | SU | 5 | 5 | Source 16 | Note 1,6 |
|  |  | 30 |  |  |  | SU | 6 | 6 | Source 16 | Note 1,6 |
|  |  | 60 |  |  |  | SU | 6 | 6 | Source 16 | Note 1,6 |
|  | AR (2 streams: Pose/control-stream + scene/video/data/voice-stream) | 10 (Pose), 30 (Scene) | 0.2 (Pose) 10 (Scene) | 250 (Pose) 60 (Scene) |  | SU | 5 | 5 | Source 16 | Note 1,6 |
|  |  |  |  |  |  | SU | 2.5 | 2.5 | Source 16 | Note 1 |
|  |  |  | 0.2 (Pose) 20 (Scene) |  |  | SU | 3.5 | 3.5 | Source 16 | Note 1,6 |
| Note 1: UE antenna configuraiton: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) Note 2: Regular slot Note 3: Full antenna (gNB uses all its N antennas and system bandwidth for receiving pose updates from a given user in the TDM) Note 4: FDM/SDM Note 5: UE antenna configuraiton: 4Tx/4Rx: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2) Note 6: DDDUU |  |  |  |  |  |  |  |  |  |  |

##### DU Scenario

###### VR/CG (Pose/control-stream)

Based on the evaluation results in Table 8.3.1.4-1, the following observations can be made.

- For FR2, Dense Urban, UL, for VR/CG (Pose/control-stream, 0.2Mbps data rate, 10ms PDB, 250 FPS):

- With Option 1 UE antenna configuration, TDD frame structure DDDSU, it is observed from Source 18 that the capacity performance is 20 UEs per cell.

- With Option 1 UE antenna configuration, TDD frame structure DDDSU, it is observed from Source 16 that the capacity performance is 7.5 UEs per cell.

- With Option 1 UE antenna configuration, TDD frame structure DDDUU, it is observed from Source 16 that the capacity performance is 18.5 UEs per cell.

- With Option 2 UE antenna configuration, TDD frame structure DDDSU, it is observed from Source 14 that the capacity performance is >30 UEs per cell.

###### AR (1 stream: Scene/video/data/voice-stream)

Based on the evaluation results in Table 8.3.1.4-1, the following observations can be made.

- For FR2, Dense Urban, UL, for AR 1-stream (scene/video/data/voice-stream, 10Mbps data rate, 30ms PDB, 60FPS):

- With Option 1 UE antenna configuration, TDD frame structure DDDSU, it is observed from Source 18 that the capacity performance is 8.3 UEs per cell.

- With Option 1 UE antenna configuration, TDD frame structure DDDSU, it is observed from Source 16 that the capacity performance is 5 UEs per cell.

- With Option 1 UE antenna configuration, TDD frame structure DDDUU, it is observed from Source 16 that the capacity performance is 9 UEs per cell.

- With Option 2 UE antenna configuration, TDD frame structure DDDSU, it is observed from Source 14 that the capacity performance is 1.29 UEs per cell.

- For FR2, Dense Urban, UL, for AR 1-stream (scene/video/data/voice-stream, 20Mbps data rate, 30ms PDB, 60FPS):

- With Option 1 UE antenna configuration, TDD frame structure DDDUU, it is observed from Source 16 that the capacity performance is 5 UEs per cell.

###### AR (2 streams: Pose/control-stream + scene/video/data/voice-stream)

Based on the evaluation results in Table 8.3.1.4-1, the following observations can be made.

- For FR2, Dense Urban, UL, for AR 2-stream (Pose/control-stream with 0.2Mbps data rate, 10ms PDB, 250FPS and scene/video/ data/voice-stream with 10Mbps data rate, 30ms PDB, 60FPS):

- With Option 1 UE antenna configuration, TDD frame structure DDDSU, it is observed from Source 16 that the capacity performance is 1.5 UEs per cell.

- With Option 1 UE antenna configuration, TDD frame structure DDDUU, it is observed from Source 16 that the capacity performance is 4.5 UEs per cell.

- For FR2, Dense Urban, UL, for AR 2-stream (Pose/control-stream with 0.2Mbps data rate, 10ms PDB, 250FPS and scene/video/ data/voice-stream with 20Mbps data rate, 30ms PDB, 60FPS):

- With Option 1 UE antenna configuration, TDD frame structure DDDUU, it is observed from Source 16 that the capacity performance is 2 UEs per cell.

##### InH Scenario

###### VR/CG (Pose/control-stream)

Based on the evaluation results in Table 8.3.1.4-1, the following observations can be made.

- For FR2, Indoor Hotspot, UL, for VR/CG (Pose/control-stream, 0.2Mbps data rate, 10ms PDB, 250 FPS) can be summarized as follows:

- With Option 1 UE antenna configuration, TDD frame structure DDDSU, it is observed from Source 18 that the capacity performance is 20 UEs per cell.

- With Option 1 UE antenna configuration, TDD frame structure DDDSU, it is observed from Source 16 that the capacity performance is 7 UEs per cell.

- With Option 1 UE antenna configuration, TDD frame structure DDDUU, it is observed from Source 16 that the capacity performance is 19 UEs per cell.

- With Option 2 UE antenna configuration, TDD frame structure DDDSU, it is observed from Source 14 that the capacity performance is 12.09 UEs per cell.

###### AR (1 stream: Scene/video/data/voice-stream)

Based on the evaluation results in Table 8.3.1.4-1, the following observations can be made.

- For FR2, Indoor Hotspot, UL, for AR 1-stream (scene/video/data/voice-stream, 10Mbps data rate, 30ms PDB, 60FPS):

- With Option 1 UE antenna configuration, TDD frame structure DDDSU, it is observed from Source 18 that the capacity performance is 8.59 UEs per cell.

- With Option 1 UE antenna configuration, TDD frame structure DDDSU, it is observed from Source 16 that the capacity performance is 5 UEs per cell.

- With Option 1 UE antenna configuration, TDD frame structure DDDUU, it is observed from Source 16 that the capacity performance is 10 UEs per cell.

- With Option 2 UE antenna configuration, TDD frame structure DDDSU, it is observed from Source 14 that the capacity performance is 1 UEs per cell.

- For FR2, Indoor Hotspot, UL, for AR 1-stream (scene/video/data/voice-stream, 20Mbps data rate, 30ms PDB, 60FPS):

- With Option 1 UE antenna configuration, TDD frame structure DDDUU, it is observed from Source 16 that the capacity performance is 6 UEs per cell.

###### AR (2 streams: Pose/control-stream + scene/video/data/voice-stream)

Based on the evaluation results in Table 8.3.1.4-1, the following observations can be made.

- For FR2, Indoor Hotspot, UL, for AR 2-stream (Pose/control-stream with 0.2Mbps data rate, 10ms PDB, 250FPS and scene/video/ data/voice-stream with 10Mbps data rate, 30ms PDB, 60FPS) can be summarized as follows:

- With Option 1 UE antenna configuration, TDD frame structure DDDSU, it is observed from Source 16 that the capacity performance is 2.5 UEs per cell.

- With Option 1 UE antenna configuration, TDD frame structure DDDUU, it is observed from Source 16 that the capacity performance is 5 UEs per cell.

- For FR2, Indoor Hotspot, UL, for AR 2-stream (Pose/control-stream with 0.2Mbps data rate, 10ms PDB, 250FPS and scene/video/ data/voice-stream with 20Mbps data rate, 30ms PDB, 60FPS) can be summarized as follows:

- With Option 1 UE antenna configuration, TDD frame structure DDDUU, it is observed from Source 16 that the capacity performance is 3.5 UEs per cell.

### Capacity Comparison for Different Parameters/Configurations

#### Capacity Comparison for Different Data-rate

This section captures the capacity performance comparison for different data-rate.

Table 8.3.2.1-1. AR/VR application capacity comparison for different data-rate

| Case | App | PDB | F(fps) | Scenario | MIMO | Capacity result (UEs/cell) with 30Mbps |  | Capacity result (UEs/cell) with 45Mbps |  | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | mean | range | mean | range |  |
| FR1 DL | AR/VR | 10ms | 60 | DU | SU | 8.22 | 5.1~10.6 | 4.58 | 1.7~6 | Note 1 |
|  |  |  |  |  | SU | 7.31 | 6.54~8.4 | 4.77 | 4.1~5 | Note2 |
|  |  |  |  |  | MU | 11.41 | 7 ~ 13.59 | 7.07 | 5.3~8.4 | Note 1 |
|  |  |  |  |  | MU | 3.9 | 3.9 | 2.4 | 2.4 | Note 2 |
|  |  |  |  | InH | SU | 7.33 | 5.2~8.5 | 4.44 | 3.27~5 |  |
|  |  |  |  |  | MU | 9.21 | 5~12 | 6.07 | 3.5~8 |  |
|  |  |  |  | UMa | SU | 6.26 | 4.4~8 | 3.62 | 1.8~4.7 | Note 1 |
|  |  |  |  |  | MU | 8.29 | 5.2~10 | 4.51 | 2.9~6 | Note 1 |
| FR2 DL |  | 10ms | 60 | DU | SU | 8.93 | 6.35~13.44 | 5.71 | 3.94~8.2 | Note 3 |
|  |  |  |  | InH | SU | 8.02 | 6.2~10.17 | 4.74 | 3.2~6.09 | Note 3 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: BS antenna parameters: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1:8,2) Note 3: UE Antenna parameters: Option 1: (M, N, P) =(1, 4, 2), 3 panels (left, right, top) |  |  |  |  |  |  |  |  |  |  |

Table 8.3.2.1-2. CG application capacity comparison for different data-rate

| Case | App | PDB | F(fps) | Scenario | MIMO | Capacity result (UEs/cell) with 8Mbps |  | Capacity result (UEs/cell) with 30Mbps |  | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | mean | range | mean | range |  |
| FR1 DL | CG | 15ms | 60 | DU | SU |  | >20~>36 | 9.89 | 6.17~13 |  |
|  |  |  |  |  | MU |  | >36~56.6 | 15.06 | 10.1~19.65 |  |
|  |  |  |  | InH | SU |  | >20~>38.7 | 8.4 | 5.96~10.5 |  |
|  |  |  |  |  | MU |  | >38.7~44.1 | 11.96 | 7.2~16.2 |  |
|  |  |  |  | UMa | SU |  | 17.5~32.9 | 8.36 | 5.4~10.33 |  |
|  |  |  |  |  | MU |  | 23.8~>36 | 11.59 | 8~14.33 |  |
| FR2 DL |  | 15ms | 60 | DU | SU |  | >20, 32.5 | 9.38 | 5.1~16.16 |  |
|  |  |  |  | InH | InH SU |  | >20, 31 | 8.94 | 6.9~11.45 |  |

Based on the evaluation results in Table 8.3.1.1-1, Table 8.3.1.3-1 and Table 8.3.2.1-1, the following observations can be made.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 10ms PDB, 60 FPS, with SU-MIMO and 64 TxRU BS antenna, it is observed that the range of capacity performance is decreased from 5.1~10.6 UEs per cell with 30Mbps to 1.7~6 UEs per cell with 45Mbps, and the mean capacity performance is decreased from 8.22 UEs per cell with 30Mbps to 4.58 UEs per cell with 45Mbps by about 44.3%.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 10ms PDB, 60 FPS, with SU-MIMO and 32 TxRU BS antenna, it is observed that the range of capacity performance is decreased from 6.54~8.4 UEs per cell with 30Mbps to 4.1~5 UEs per cell with 45Mbps, and the mean capacity performance is decreased from 7.31 UEs per cell with 30Mbps to 4.77 UEs per cell with 45Mbps by about 34.7%.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 10ms PDB, 60 FPS, with MU-MIMO and 64 TxRU BS antenna, it is observed that the range of capacity performance is decreased from 7~13.59 UEs per cell with 30Mbps to 5.3~8.4 UEs per cell with 45Mbps, and the mean capacity performance is decreased from 11.41 UEs per cell with 30Mbps to 7.07 UEs per cell with 45Mbps by about 38.0%.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 10ms PDB, 60 FPS, with MU-MIMO and 32 TxRU BS antenna, it is observed from Source 11 that the capacity performance is decreased from 3.9 UEs per cell with 30Mbps to 2.4 UEs per cell with 45Mbps by about 45.6%.

- For FR1, Indoor Hotspot, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 10ms PDB, 60 FPS, with SU-MIMO, it is observed that the range of capacity performance is decreased from 5.2~8.5 UEs per cell with 30Mbps to 3.27~5 UEs per cell with 45Mbps, and the mean capacity performance is decreased from 7.33 UEs per cell with 30Mbps to 4.44 UEs per cell with 45Mbps by about 39.4%.

- For FR1, Indoor Hotspot, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 10ms PDB, 60 FPS, with SU-MIMO, it is observed that the range of capacity performance is decreased from 5~12 UEs per cell with 30Mbps to 3.5~8 UEs per cell with 45Mbps, and the mean capacity performance is decreased from 9.21 UEs per cell with 30Mbps to 6.07 UEs per cell with 45Mbps by about 34.1%.

- For FR1, Urban Macro, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 10ms PDB, 60 FPS, with SU-MIMO, it is range of observed that the capacity performance is decreased from 4.4~8 UEs per cell with 30Mbps to 1.8~4.7 UEs per cell with 45Mbps, and the mean capacity performance is decreased from 6.26 UEs per cell with 30Mbps to 3.62 UEs per cell with 45Mbps by about 42.2%.

- For FR1, Urban Macro, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 10ms PDB, 60 FPS, with SU-MIMO, it is observed that the range of capacity performance is decreased from 5.2~10 UEs per cell with 30Mbps to 2.9~6 UEs per cell with 45Mbps, and the mean capacity performance is decreased from 8.29 UEs per cell with 30Mbps to 4.51 UEs per cell with 45Mbps by about 45.6%.

- For FR2, Dense Urban, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 10ms PDB, 60 FPS, with SU-MIMO, it is observed that the range of capacity performance is decreased from 6.35~13.44 UEs per cell with 30Mbps to 3.94~8.2 UEs per cell with 45Mbps, and the mean capacity performance is decreased from 8.93 UEs per cell with 30Mbps to 5.71 UEs per cell with 45Mbps by about 36.1%.

- For FR2, Indoor Hotspot, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 10ms PDB, 60 FPS, with SU-MIMO, it is observed that the range of capacity performances are decreased from 6.2~10.17 UEs per cell with 30Mbps to 3.2~6.09 UEs per cell with 45Mbps, and the mean capacity performance is decreased from 8.02 UEs per cell with 30Mbps to 4.74 UEs per cell with 45Mbps by about 40.9%.

Based on the evaluation results in Table 8.3.1.1-1, Table 8.3.1.3-1 and Table 8.3.2.1-2, the following observations can be made.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for CG traffic model, 15ms PDB, 60 FPS, with SU-MIMO and 64 TxRU BS antenna, it is observed that the range of capacity performance is decreased from >20~>36 UEs per cell with 8Mbps to 6.17~13 UEs per cell with 30Mbps.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for CG traffic model, 15ms PDB, 60 FPS, with MU-MIMO and 64 TxRU BS antenna, it is observed that the range of capacity performance is decreased from >36~56.6 UEs per cell with 8Mbps to 10.1~19.65 UEs per cell with 30Mbps.

- For FR1, Indoor Hotspot, DL, with 100MHz bandwidth for CG traffic model, 15ms PDB, 60 FPS, with SU-MIMO, it is observed that the range of capacity performance is decreased from >20~>38.7 UEs per cell with 8Mbps to 5.96~10.5 UEs per cell with 30Mbps.

- For FR1, Indoor Hotspot, DL, with 100MHz bandwidth for CG traffic model, 15ms PDB, 60 FPS, with MU-MIMO, it is observed that the range of capacity performance is decreased from >38.7~44.1 UEs per cell with 8Mbps to 7.2~16.2 UEs per cell with 30Mbps.

- For FR1, Urban Macro, DL, with 100MHz bandwidth for CG traffic model, 15ms PDB, 60 FPS, with SU-MIMO, it is observed that the range of capacity performance is decreased from 17.5~32.9 UEs per cell with 8Mbps to 5.4~10.33 UEs per cell with 30Mbps.

- For FR1, Urban Macro, DL, with 100MHz bandwidth for CG traffic model, 15ms PDB, 60 FPS, with MU-MIMO, it is observed that the range of capacity performance is decreased from 23.8~>36 UEs per cell with 8Mbps to 8~14.33 UEs per cell with 30Mbps.

- For FR2, Dense Urban, DL, with 100MHz bandwidth for CG traffic model, 15ms PDB, 60 FPS, with SU-MIMO, it is observed that the range of capacity performance is decreased from >20~32.5 UEs per cell with 8Mbps to 5.1~16.16 UEs per cell with 30Mbps.

- For FR2, Dense Urban, DL, with 400MHz bandwidth for CG traffic model, 15ms PDB, 60 FPS, with SU-MIMO, it is observed from Source 16 that the mean capacity performance is decreased from >30 UEs per cell with 8Mbps to 25 UEs per cell with 30Mbps.

- For FR2, Indoor Hotspot, DL, with 100MHz bandwidth for CG traffic model, 15ms PDB, 60 FPS, with SU-MIMO, it is observed that the range of capacity performance is decreased from >20~31 UEs per cell with 8Mbps to 6.9~11.45 UEs per cell with 30Mbps.

Based on the evaluation results in Table 8.3.1.4-1, the observations for capacity performance evaluation with AR 1-stream scene/video/data/voice-stream for different data-rate can be summarized as follows:

- For FR2, Dense Urban, UL, with 100MHz bandwidth for AR 1-stream scene/video/data/voice-stream, it is observed from Source 16 that the mean capacity performance is decreased from 9 UEs per cell with 10Mbps to 5 UEs per cell with 20Mbps by about 44.44%.

- For FR2, Indoor Hotspot, UL, with 100MHz bandwidth for AR 1-stream scene/video/data/voice-stream, it is observed from Source 16 that the mean capacity performance is decreased from 10 UEs per cell with 10Mbps to 6 UEs per cell with 20Mbps by about 40%.

Based on the evaluation results in Table 8.3.1.4-1, the observations for capacity performance evaluation with AR 2-stream pose/control-stream for different data-rate can be summarized as follows:

- For FR2, Dense Urban, UL, with 100MHz bandwidth for AR 2-stream pose/control-stream and scene/video/ data/voice-stream, it is observed from Source 16 that the mean capacity performance is decreased from 4.5 UEs per cell with video-stream 10Mbps to 2 UEs per cell with video-stream 20Mbps by about 55.56%.

- For FR2, Indoor Hotspot, UL, with 100MHz bandwidth for AR 2-stream pose/control-stream and scene/video/ data/voice-stream, it is observed from Source 16 that the mean capacity performance is decreased from 5 UEs per cell with video-stream 10Mbps to 3.5 UEs per cell with video-stream 20Mbps by about 30%.

#### Capacity Comparison for Different PDB/PER Values

This section captures the capacity performance comparison for different PDB/PER values. The definitions of PDB/PER refer to section 5.1.1.3 and 5.1.1.4.

Table 8.3.2.2-1. Single-stream capacity comparison for different PDB values

| Case | R | F(fps) | Scenario | MIMO | PDB 1 | Capacity result 1 (UEs/cell) |  | PDB 2 | Capacity result 2 (UEs/cell) |  | Source | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | mean | range |  | mean | range |  |  |
| FR1 DL | 30Mbps | 60 | DU | SU | 10ms | 7.72 | 4.05~10.6 | 15ms | 9.34 | 5.57~13 | Source 3, Source 4, Source 5, Source 6, Source 7, Source 8, Source 9, Source 10, Source 14, Source 15, Source 16, Source 17, Source 18 Source 19 |  |
|  |  |  |  | MU | 7ms | 7.35 | 6.3~8.4 | 10ms | 11.9 | 11.5~12.3 | Source 8, Source 9 |  |
|  |  |  |  |  | 10ms | 11.9 | 11.5~12.3 | 13ms | 14.65 | 14.6~14.7 | Source 8, Source 9 |  |
|  |  |  |  |  | 10ms | 10.19 | 3.9~13.59 | 15ms | 13.25 | 5~19.65 | Source 4, Source 6, Source 7, Source 8, Source 9, Source 10, Source 11, Source 16, Source 18, Source 20 |  |
|  |  |  | InH | SU | 10ms | 6.97 | 4.85~8.5 | 15ms | 8.53 | 5.96~10.5 | Source 6, Source 7, Source 12, Source 14, Source 15, Source 16, Source 18, Source 19 |  |
|  |  |  |  | MU | 7ms | 8 |  | 10ms | 12 |  | Source 3 |  |
|  |  |  |  |  | 10ms | 9.21 | 5~12 | 15ms | 11.96 | 7.2~16.2 | Source 3, Source 6, Source 7, Source 11, Source 16, Source 18, Source 20 |  |
|  |  |  | Uma | SU | 10ms | 5.85 | 2.98~8 | 15ms | 7.83 | 4.08~10.33 | Source 4, Source 5, Source 7, Source 8, Source 9, Source 14, Source 16, Source 18, Source 20 |  |
|  |  |  |  | MU | 10ms | 8.40 | 5.2~10 | 15ms | 11.59 | 8~14.33 | Source 7, Source 8, Source 9, Source 16, Source 18, Source 20 |  |
|  | 45Mbps | 60 | DU | SU | 10ms | 5 | 4.4-5.4 | 15ms | 6.33 | 6.3~6.4 | Source 17 |  |
| FR2 DL | 30Mbps | 60 | DU | SU | 10ms | 8.20 | 4.2~13.44 | 15ms | 9.70 | 5.1~16.16 | Source 7, Source 14, Source 15, Source 16, Source 18 |  |
|  |  |  |  |  | 10ms | 30 |  | 15ms | 32.5 |  | Source 16 | Note 1 |
|  |  |  | InH | SU | 10ms | 8.74 | 7~10.17 | 15ms | 9.95 | 7.5~11.45 | Source 14, Source 15, Source 16, Source 18, Source 20 |  |
|  |  |  |  |  | 10ms | 34 |  | 15ms | 36 |  | Source 16 | Note 1 |
| FR1 UL | 10Mbps | 60 | DU | MU | 10ms | <1 |  | 30ms | 8.1 |  | Source 9 |  |
|  |  |  |  |  | 15ms | 5.4 |  | 30ms | 8.1 |  | Source 9 |  |
|  |  |  |  |  | 30ms | 8.1 |  | 60ms | 8.3 |  | Source 9 |  |
| FR2 UL | 20Mbps | 60 | DU | SU | 15 ms | 3.5 |  | 30ms | 5 |  | Source 16 |  |
|  |  |  | InH |  | 15 ms | 5 |  | 30ms | 6 |  | Source 16 |  |
| Note1: 400MHz bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |

Table 8.3.2.2-2. Single-stream capacity comparison for different PER values

| Case | R | PDB | F(fps) | Scenario | MIMO | PER 1 | Capacity result 1 (UEs/cell) | PER 2 | Capacity result 2 (UEs/cell) | Source | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR1 DL | 30Mbps | 10ms | 60 | DU | MU | 0.5% | 9.9 | 1% | 11.5 | Source 9 |  |
|  |  |  |  |  |  | 1% | 11.5 | 5% | 16.8 | Source 9 |  |
| FR1 UL | 10Mbps | 30ms | 60 | DU | MU | 1% | 8.1 | 5% | 8.3 | Source 9 |  |
|  |  |  |  |  |  | 1% | 8.1 | 10% | 8.4 | Source 9 |  |

##### Single-stream traffic model

Based on the evaluation results in Table 8.3.2.2-2, the following observations can be made.

- For FR1, Dense Urban, DL, with single stream traffic model, 30Mbps, 60FPS, with MU-MIMO, with PER decrease from 1% to 0.5%, it is observed from Source 9 that the mean capacity performance is decreased from 11.5 UEs per cell to 9.9 UEs per cell by about 13.9%.

- For FR1, Dense Urban, DL, with single stream traffic model, 30Mbps, 60FPS, with MU-MIMO, with PER increase from 1% to 5%, it is observed from Source 9 that the mean capacity performance is increased from 11.5 UEs per cell to 16.8 UEs per cell by about 46.1%.

Based on the evaluation results in Table 8.3.2.2-1, the following observations can be made.

- For FR1, Dense Urban, DL, with single stream traffic model, 30Mbps, 60FPS, with SU-MIMO, with PDB increase from 10ms to 15ms, it is observed from Source 3, Source 4, Source 5, Source 6, Source 7, Source 8, Source 9, Source 10, Source 14, Source 15, Source 16, Source 17, Source 18 Source 19 that the range of capacity performance is increased from 4.05~10.6 UEs per cell to 5.57~13 UEs per cell and the mean capacity performance is increased from 7.72 UEs per cell to 9.34 UEs per cell by about 21.0%.

- For FR1, Dense Urban, DL, with single stream traffic model, 30Mbps, 60FPS, with MU-MIMO, with PDB increase from 10ms to 15ms, it is observed from Source 4, Source 6, Source 7, Source 8, Source 9, Source 10, Source 11, Source 16, Source 18, Source 20 that the range of capacity performance is increased from 3.9~13.59 UEs per cell to 5~19.65 UEs per cell and the mean capacity performance is increased from 10.19 UEs per cell to 13.25 UEs per cell by about 30.0 %.

- For FR1, Indoor Hotspot, DL, with single stream traffic model, 30Mbps, 60FPS, with SU-MIMO, with PDB increase from 10ms to 15ms, it is observed from Source 6, Source 7, Source 12, Source 14, Source 15, Source 16, Source 18, Source 19 that the range of capacity performance is increased from 4.85~8.5 UEs per cell to 5.96~10.5 UEs per cell and the mean capacity performance is increased from 6.97 UEs per cell to 8.53 UEs per cell by about 22.4%.

- For FR1, Indoor Hotspot, DL, with single stream traffic model, 30Mbps, 60FPS, with MU-MIMO, with PDB increase from 10ms to 15ms, it is observed from Source 3, Source 6, Source 7, Source 11, Source 16, Source 18, Source 20 that the range of capacity performance is increased from 5~12 UEs per cell to 7.2~16.2 UEs per cell and the mean capacity performance is increased from 9.21 UEs per cell to 11.96 UEs per cell by about 29.9%.

- For FR1, Urban Macro, DL, with single stream traffic model, 30Mbps, 60FPS, with SU-MIMO, with PDB increase from 10ms to 15ms, it is observed from Source 4, Source 5, Source 7, Source 8, Source 9, Source 14, Source 16, Source 18, Source 20 that the range of capacity performance is increased from 2.98~8 UEs per cell to 4.08~10.33 UEs per cell and the mean capacity performance is increased from 5.85 UEs per cell to 7.83 UEs per cell by about 33.9%.

- For FR1, Urban Macro, DL, with single stream traffic model, 30Mbps, 60FPS, with MU-MIMO, with PDB increase from 10ms to 15ms, it is observed from Source 7, Source 8, Source 9, Source 16, Source 18, Source 20 that the range of capacity performance is increased from 5.2~10 UEs per cell to 8~14.33 UEs per cell and the mean capacity performance is increased from 8.40 UEs per cell to 11.59 UEs per cell by about 38.0%.

- For FR1, Dense Urban, DL, with single stream traffic model, 30Mbps, 60FPS, with MU-MIMO, with PDB decrease from 10ms to 7ms, it is observed from Source 8, Source 9 that the range of capacity performance is decreased from 11.5~12.3 UEs per cell to 6.3~8.4 UEs per cell and the mean capacity performance is decreased from 11.90 UEs per cell to 7.35 UEs per cell by about 38.2%.

- For FR1, Dense Urban, DL, with single stream traffic model, 30Mbps, 60FPS, with MU-MIMO, with PDB increase from 10ms to 13ms, it is observed from Source 8, Source 9 that the range of capacity performance is decreased from 11.5~12.3 UEs per cell to 14.6~14.7 UEs per cell and the mean capacity performance is increased from 11.9 UEs per cell to 14.65 UEs per cell by about 23.1%.

- For FR1, Dense Urban, DL, with single stream traffic model, 45Mbps, 60FPS, with SU-MIMO, with PDB increase from 10ms to 15ms, it is observed from Source 17 that the range of capacity performance is increased from 4.4~5.4 UEs per cell to 6.3~6.4 UEs per cell and the mean capacity performance is increased from 5 UEs per cell to 6.33 UEs per cell by about 21.0%.

- For FR1, Indoor Hotspot, DL, with single stream traffic model, 30Mbps, 60FPS, with MU-MIMO, with PDB decrease from 10ms to 7ms, it is observed from Source 3 that the mean capacity performance is decreased from 12 UEs per cell to 8 UEs per cell by about 33.3%.

- For FR1, Dense Urban, UL, with AR single-stream (Scene/video/data/ audio -stream, 10Mbps, 60FPS), with PDB decrease from 30ms to 10ms, it is observed from Source 9 that the mean capacity performance is decreased from 8.1 UEs per cell to <1 UEs per cell by about 87.7%.

- For FR1, Dense Urban, UL, with AR single-stream (Scene/video/data/ audio -stream, 10Mbps, 60FPS), with PDB decrease from 30ms to 15ms, it is observed from Source 9 that the mean capacity performance is decreased from 8.1 UEs per cell to 5.4 UEs per cell by about 33.3%.

- For FR1, Dense Urban, UL, with AR single-stream (Scene/video/data/ audio -stream, 10Mbps, 60FPS), with PDB increase from 30ms to 60ms, it is observed from Source 9 that the mean capacity performance is increased from 8.1 UEs per cell to 8.3 UEs per cell by about 2.5%.

- For FR2, Dense Urban, DL, with single stream traffic model, 30Mbps, 60FPS, with PDB increase from 10ms to 15ms, it is observed from Source 7, Source 14, Source 15, Source 16, Source 18 that the range of capacity performance is increased from 4.2~13.44 UEs per cell to 5.1~16.16 UEs per cell and the mean capacity performance is increased from 8.20 UEs per cell to 9.70 UEs per cell by about 18.3%.

- For FR2, Dense Urban, DL, with single stream traffic model, 30Mbps, 60FPS, with 400MHz bandwidth, with PDB increase from 10ms to 15ms, it is observed from Source 16 that the mean capacity performance is increased from 30 UEs per cell to 32.5 UEs per cell by about 8.3%.

- For FR2, Indoor Hotspot, DL, with single stream traffic model, 30Mbps, 60FPS, with PDB increase from 10ms to 15ms, it is observed from Source 14, Source 15, Source 16, Source 18, Source 20 that the range of capacity performance is increased from 7~10.17 UEs per cell to 7.5~11.45 UEs per cell and the mean capacity performance is increased from 8.74 UEs per cell to 9.95 UEs per cell by about 13.8%.

- For FR2, Indoor Hotspot, DL, with single stream traffic model, 30Mbps, 60FPS, with 400MHz bandwidth, with PDB increase from 10ms to 15ms, it is observed from Source 16 that the mean capacity performance is increased from 34 UEs per cell to 36 UEs per cell by about 5.9%.

- For FR2, Dense Urban, UL, with AR single-stream (Scene/video/data/ audio -stream, 20Mbps, 60FPS), with PDB decrease from 30ms to 15ms, it is observed from Source 16 that the mean capacity performance is decreased from 5 UEs per cell to 3.5 UEs per cell by about 30.0%.

- For FR2, Dense Urban, UL, with AR single-stream (Scene/video/data/ audio -stream, 20Mbps, 60FPS), with PDB increase from 30ms to 60ms, it is observed from Source 16 that the mean capacity performance is not affected.

- For FR2, Indoor Hotspot, UL, with AR single-stream (Scene/video/data/ audio -stream, 20Mbps, 60FPS), with PDB decrease from 30ms to 15ms, it is observed from Source 16 that the mean capacity performance is decreased from 6 UEs per cell to 5 UEs per cell by about 16.7%.

- For FR2, Indoor Hotspot, UL, with AR single-stream (Scene/video/data/ audio -stream, 20Mbps, 60FPS), with PDB increase from 30ms to 60ms, it is observed from Source 16 that the mean capacity performance is not affected.

##### Multi-stream traffic model

The observations for capacity performance evaluation with multi-stream traffic model for different PDB/PER values can be summarized as follows:

- For FR1, Dense Urban, DL, with VR/AR GOP-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with alphaα = 1.5, with I_PER = 1% and P_PER = 1%, with P_PDB = 10ms and I_PDB increase from 10ms to 15ms, it is identified observed from Source 18 that that the mean capacity performance is increased from 6.74 UEs per cell to 12.58 UEs per cell by about 31.7%.

- For FR1, Dense Urban, DL, with VR/AR GOP-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with alphaα = 2, with I_PER = 1% and P_PER = 1%, with P_PDB = 10ms and I_PDB increase from 10ms to 15ms, it is identified observed from Source 9 that that the mean capacity performance is increased from 6.7 UEs per cell to 9.1 UEs per cell by about 35.82%.

- For FR1, Dense Urban, DL, with VR/AR GOP-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with alphaα = 2, with I_PER = 1% and P_PER = 1%, with P_PDB = 10ms and I_PDB increase from 10ms to 15ms, it is identified observed from Source 18 that that the mean capacity performance is increased from 5.2 UEs per cell to 10.06 UEs per cell by about 93.46%.

- For FR1, Dense Urban, DL, with VR/AR GOP-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with alphaα = 2, with I_PER = 1% and P_PER = 1%, with P_PDB = 10ms and I_PDB increase from 10ms to 15ms, it is identified observed from Source 14 that that the mean capacity performance is increased from 6 UEs per cell to 10 UEs per cell by about 66.67%.

- For FR1, Dense Urban, DL, with VR/AR GOP-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with alphaα = 3, with I_PER = 1% and P_PER = 1%, with P_PDB = 10ms and I_PDB increase from 10ms to 15ms, it is identified observed from Source 14 that that the mean capacity performance is increased from 2.21 UEs per cell to 5.73 UEs per cell by about 43.7%.

- For FR1, Dense Urban, DL, with VR/AR GOP-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with alphaα = 1.5, with I_PER = 1% and P_PER = 1%, with I_PDB = 15ms and P_PDB decrease from 10ms to 9ms, it is identified observed from Source 18 that that the mean capacity performance is decreased from 12.58 UEs per cell to 12.39 UEs per cell by about 2.3%.

- For FR1, Dense Urban, DL, with VR/AR GOP-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with alphaα = 2, with I_PER = 1% and P_PER = 1%, with I_PDB = 15ms and P_PDB decrease from 10ms to 9ms, it is identified observed from Source 9 that that the mean capacity performance is decreased from 9.1 UEs per cell to 8.8 UEs per cell by about 3.30%.

- For FR1, Dense Urban, DL, with VR/AR GOP-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with alphaα = 2, with I_PER = 1% and P_PER = 1%, with I_PDB = 15ms and P_PDB decrease from 10ms to 9ms, it is identified observed from Source 18 that that the mean capacity performance is decreased from 10.06 UEs per cell to 9.19 UEs per cell by about 8.65%.

- For FR1, Dense Urban, DL, with VR/AR GOP-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with alphaα = 3, with I_PER = 1% and P_PER = 1%, with I_PDB = 15ms and P_PDB decrease from 10ms to 9ms, it is identified observed from Source 18 that that the mean capacity performance is decreased from 5.73 UEs per cell to 5.69 UEs per cell by about 2.3%.

- For FR1, Dense Urban, DL, with VR/AR GOP-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 1.5, with I_PDB = 10ms and P_PDB = 10ms, with I_PER = 1% and P_PER increase from 1% to 5%, it is observed from Source 18 that capacity performances are both 6.74 UEs per cell.

- For FR1, Dense Urban, DL, with VR/AR GOP-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 2, with I_PDB = 10ms and P_PDB = 10ms, with I_PER = 1% and P_PER increase from 1% to 5%, it is observed from Source 9 that capacity performances are both 6.7 UEs per cell.

- For FR1, Dense Urban, DL, with VR/AR GOP-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 2, with I_PDB = 10ms and P_PDB = 10ms, with I_PER = 1% and P_PER increase from 1% to 5%, it is observed from Source 20 that capacity performances are 10.8~10.9 UEs per cell.

- For FR1, Dense Urban, DL, with VR/AR GOP-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 2, with I_PDB = 10ms and P_PDB = 10ms, with I_PER = 1% and P_PER increase from 1% to 5%, it is observed from Source 18 that capacity performances are both 5.2 UEs per cell.

- For FR1, Dense Urban, DL, with VR/AR GOP-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 2, with I_PDB = 10ms and P_PDB = 10ms, with I_PER = 1% and P_PER increase from 1% to 5%, it is observed from Source 14 that capacity performances are both 6 UEs per cell.

- For FR1, Dense Urban, DL, with VR/AR GOP-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 3, with I_PDB = 10ms and P_PDB = 10ms, with I_PER = 1% and P_PER increase from 1% to 5%, it is observed from Source 18 that capacity performances are both 2.21 UEs per cell.

- For FR1, Dense Urban, DL, with VR/AR GOP-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 1.5, with I_PDB = 10ms and P_PDB = 10ms, with P_PER = 5% and I_PER decrease from 1% to 0.5%, it is observed from Source 18 that the mean capacity performance is decreased from 6.74 UEs per cell to 6.39 UEs per cell by about 7.3%.

- For FR1, Dense Urban, DL, with VR/AR GOP-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 2, with I_PDB = 10ms and P_PDB = 10ms, with P_PER = 5% and I_PER decrease from 1% to 0.5%, it is observed from Source 9 that the mean capacity performance is decreased from 6.7 UEs per cell to 6 UEs per cell by about 10.45%.

- For FR1, Dense Urban, DL, with VR/AR GOP-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 2, with I_PDB = 10ms and P_PDB = 10ms, with P_PER = 5% and I_PER decrease from 1% to 0.5%, it is observed from Source 18 that the mean capacity performance is decreased from 5.2 UEs per cell to 4.74 UEs per cell by about 8.85%.

- For FR1, Dense Urban, DL, with VR/AR GOP-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 2, with I_PDB = 10ms and P_PDB = 10ms, with P_PER = 5% and I_PER decrease from 1% to 0.5%, it is observed from Source 14 that the mean capacity performance is decreased from 6 UEs per cell to 2 UEs per cell by about 66.67%.

- For FR1, Dense Urban, DL, with VR/AR GOP-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 3, with I_PDB = 10ms and P_PDB = 10ms, with P_PER = 5% and I_PER decrease from 1% to 0.5%, it is observed from Source 18 that the mean capacity performance is decreased from 2.21 UEs per cell to 2.09 UEs per cell by about 11.4%.

- For FR1, Dense Urban, DL, with VR/AR GOP-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 2, with I_PDB = 10ms and P_PDB = 10ms, with P_PER = 1% and I_PER increase from 1% to 10%, it is observed from Source 20 that the mean capacity performance is increased from 10.8 UEs per cell to 12.2 UEs per cell by about 12.96%.

- For FR1, Dense Urban, DL, with VR/AR GOP-Based I/P Frame multi-stream traffic model, 45Mbps, 60FPS, with α = 2, with I_PER = 1% and P_PER = 1%, with P_PDB = 10ms and I_PDB increase from 10ms to 15ms, it is observed from Source 14 that the mean capacity performance is increased from 2 UEs per cell to 4 UEs per cell by about 100.

- For FR1, Dense Urban, DL, with VR/AR GOP-Based I/P Frame multi-stream traffic model, 45Mbps, 60FPS, with α = 2, with I_PDB = 10ms and P_PDB = 10ms, with I_PER = 1% and P_PER increase from 1% to 5%, it is observed from Source 14 that capacity performances are both 2 UEs per cell.

- For FR1, Dense Urban, DL, with VR/AR Slice-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 1.5, with I_PER = 1% and P_PER = 1%, with P_PDB = 10ms and I_PDB increase from 10ms to 15ms, it is observed from Source 18 that the mean capacity performance is increased from 13.78 UEs per cell to 13.93 UEs per cell by about 1.09%.

- For FR1, Dense Urban, DL, with VR/AR Slice-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 2, with I_PER = 1% and P_PER = 1%, with P_PDB = 10ms and I_PDB increase from 10ms to 15ms, it is observed from Source 18 that the mean capacity performance is increased from 13.69 UEs per cell to 13.73 UEs per cell by about 0.29%.

- For FR1, Dense Urban, DL, with VR/AR Slice-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 3, with I_PER = 1% and P_PER = 1%, with P_PDB = 10ms and I_PDB increase from 10ms to 15ms, it is observed from Source 18 that the mean capacity performance is increased from 13.77 UEs per cell to 13.84 UEs per cell by about 0.51%.

- For FR1, Dense Urban, DL, with VR/AR Slice-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 1.5, with I_PER = 1% and P_PER = 1%, with I_PDB = 15ms and P_PDB decrease from 10ms to 9ms, it is observed from Source 18 that the mean capacity performance is increased from 13.93 UEs per cell to 13.27 UEs per cell by about 4.74%.

- For FR1, Dense Urban, DL, with VR/AR Slice-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 2, with I_PER = 1% and P_PER = 1%, with I_PDB = 15ms and P_PDB decrease from 10ms to 9ms, it is observed from Source 18 that the mean capacity performance is increased from 13.73 UEs per cell to 13.36 UEs per cell by about 2.69%.

- For FR1, Dense Urban, DL, with VR/AR Slice-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 3, with I_PER = 1% and P_PER = 1%, with I_PDB = 15ms and P_PDB decrease from 10ms to 9ms, it is observed from Source 18 that the mena capacity performance is increased from 13.84 UEs per cell to 13.46 UEs per cell by about 2.75%.

- For FR1, Dense Urban, DL, with VR/AR Slice-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 1.5, with I_PDB = 10ms and P_PDB = 10ms, with I_PER = 1% and P_PER increase from 1% to 5%, it is observed from Source 18 that the mean capacity performance is increased from 13.78 UEs per cell to 16.74 UEs per cell by about 21.48%.

- For FR1, Dense Urban, DL, with VR/AR Slice-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 2, with I_PDB = 10ms and P_PDB = 10ms, with I_PER = 1% and P_PER increase from 1% to 5%, it is observed from Source 9 that the mean capacity performance is increased from 14.9 UEs per cell to 17.3 UEs per cell by about 16.11%.

- For FR1, Dense Urban, DL, with VR/AR Slice-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 2, with I_PDB = 10ms and P_PDB = 10ms, with I_PER = 1% and P_PER increase from 1% to 5%, it is observed from Source 20 that the mean capacity performance is increased from 12.7 UEs per cell to 14.6 UEs per cell by about 14.96%.

- For FR1, Dense Urban, DL, with VR/AR Slice-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 2, with I_PDB = 10ms and P_PDB = 10ms, with I_PER = 1% and P_PER increase from 1% to 5%, it is observed from Source 18 that the mean capacity performance is increased from 13.69 UEs per cell to 16.84 UEs per cell by about 23.01%.

- For FR1, Dense Urban, DL, with VR/AR Slice-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 3, with I_PDB = 10ms and P_PDB = 10ms, with I_PER = 1% and P_PER increase from 1% to 5%, it is observed from Source 18 that the mean capacity performance is increased from 13.77 UEs per cell to 16.89 UEs per cell by about 22.66%.

- For FR1, Dense Urban, DL, with VR/AR Slice-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 1.5, with I_PDB = 10ms and P_PDB = 10ms, with P_PER = 5% and I_PER decrease from 1% to 0.5%, it is observed from Source 18 that capacity performances are both 16.74 UEs per cell.

- For FR1, Dense Urban, DL, with VR/AR Slice-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 2, with I_PDB = 10ms and P_PDB = 10ms, with P_PER = 5% and I_PER decrease from 1% to 0.5%, it is observed from Source 9 that the mean capacity performance is decreased from 17.3 UEs per cell to 15.7 UEs per cell by about 9.25%.

- For FR1, Dense Urban, DL, with VR/AR Slice-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 2, with I_PDB = 10ms and P_PDB = 10ms, with P_PER = 5% and I_PER decrease from 1% to 0.5%, it is observed from Source 18 that the mean capacity performance is decreased from 16.84 UEs per cell to 16.59 UEs per cell by about 1.48%.

- For FR1, Dense Urban, DL, with VR/AR Slice-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 3, with I_PDB = 10ms and P_PDB = 10ms, with P_PER = 5% and I_PER decrease from 1% to 0.5%, it is observed from Source 18 that capacity performances are both 16.89 UEs per cell.

- For FR2, Indoor Hotspot, DL, with VR/AR GOP-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 1.5, with I_PER = 1% and P_PER = 1%, with P_PDB = 10ms and I_PDB increase from 10ms to 15ms, it is observed from Source 18 that the mean capacity performance is increased from 5.37 UEs per cell to 7.07 UEs per cell by about 31.7%.

- For FR2, Indoor Hotspot, DL, with VR/AR GOP-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 2, with I_PER = 1% and P_PER = 1%, with P_PDB = 10ms and I_PDB increase from 10ms to 15ms, it is observed from Source 18 that the mean capacity performance is increased from 3.53 UEs per cell to 5.23 UEs per cell by about 48.2%.

- For FR2, Indoor Hotspot, DL, with VR/AR GOP-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 3, with I_PER = 1% and P_PER = 1%, with P_PDB = 10ms and I_PDB increase from 10ms to 15ms, it is observed from Source 18 that the mean capacity performance is increased from 2.29 UEs per cell to 3.29 UEs per cell by about 43.7%.

- For FR2, Indoor Hotspot, DL, with VR/AR GOP-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 1.5, with I_PER = 1% and P_PER = 1%, with I_PDB = 15ms and P_PDB decrease from 10ms to 9ms, it is observed from Source 18 that the mean capacity performance is decreased from 7.07 UEs per cell to 6.91 UEs per cell by about 2.3%.

- For FR2, Indoor Hotspot, DL, with VR/AR GOP-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 2, with I_PER = 1% and P_PER = 1%, with I_PDB = 15ms and P_PDB decrease from 10ms to 9ms, it is observed from Source 18 that the mean capacity performance is decreased from 5.23 UEs per cell to 4.99 UEs per cell by about 4.6%.

- For FR2, Indoor Hotspot, DL, with VR/AR GOP-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 3, with I_PER = 1% and P_PER = 1%, with I_PDB = 15ms and P_PDB decrease from 10ms to 9ms, it is observed from Source 18 that capacity performances are both 3.29 UEs per cell.

- For FR2, Indoor Hotspot, DL, with VR/AR GOP-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 1.5, with I_PDB = 10ms and P_PDB = 10ms, with I_PER = 1% and P_PER increase from 1% to 5%, it is observed from Source 18 that the mean capacity performance is increased from 5.37 UEs per cell to 5.43 UEs per cell by about 1.1%.

- For FR2, Indoor Hotspot, DL, with VR/AR GOP-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 2, with I_PDB = 10ms and P_PDB = 10ms, with I_PER = 1% and P_PER increase from 1% to 5%, it is observed from Source 18 that the mean capacity performance is increased from 3.53 UEs per cell to 3.87 UEs per cell by about 9.6%.

- For FR2, Indoor Hotspot, DL, with VR/AR GOP-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 3, with I_PDB = 10ms and P_PDB = 10ms, with I_PER = 1% and P_PER increase from 1% to 5%, it is observed from Source 18 that capacity performances are both 2.29 UEs per cell.

- For FR2, Indoor Hotspot, DL, with VR/AR GOP-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 1.5, with I_PDB = 10ms and P_PDB = 10ms, with P_PER = 5% and I_PER decrease from 1% to 0.5%, it is observed from Source 18 that the mean capacity performance is decreased from 5.37 UEs per cell to 4.98 UEs per cell by about 7.3%.

- For FR2, Indoor Hotspot, DL, with VR/AR GOP-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 2, with I_PDB = 10ms and P_PDB = 10ms, with P_PER = 5% and I_PER decrease from 1% to 0.5%, it is observed from Source 18 that the mean capacity performance is decreased from 3.53 UEs per cell to 2.73 UEs per cell by about 22.7%.

- For FR2, Indoor Hotspot, DL, with VR/AR GOP-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 3, with I_PDB = 10ms and P_PDB = 10ms, with P_PER = 5% and I_PER decrease from 1% to 0.5%, it is observed from Source 18 that the mean capacity performance is decreased from 2.29 UEs per cell to 2.03 UEs per cell by about 11.4%.

- For FR2, Indoor Hotspot, DL, with VR/AR Slice-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 1.5, with I_PER = 1% and P_PER = 1%, with P_PDB = 10ms and I_PDB increase from 10ms to 15ms, it is observed from Source 18 that capacity performance is 8.23~8.24 UEs per cell.

- For FR2, Indoor Hotspot, DL, with VR/AR Slice-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 2, with I_PER = 1% and P_PER = 1%, with P_PDB = 10ms and I_PDB increase from 10ms to 15ms, it is observed from Source 18 that capacity performances are both 8.24 UEs per cell.

- For FR2, Indoor Hotspot, DL, with VR/AR Slice-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 3, with I_PER = 1% and P_PER = 1%, with P_PDB = 10ms and I_PDB increase from 10ms to 15ms, it is observed from Source 18 that capacity performances are 8.23~8.28 UEs per cell.

- For FR2, Indoor Hotspot, DL, with VR/AR Slice-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 1.5, with I_PER = 1% and P_PER = 1%, with I_PDB = 15ms and P_PDB decrease from 10ms to 9ms, it is observed from Source 18 that the mean capacity performance is decreased from 8.24 UEs per cell to 8.14 UEs per cell by about 1.2%.

- For FR2, Indoor Hotspot, DL, with VR/AR Slice-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 2, with I_PER = 1% and P_PER = 1%, with I_PDB = 15ms and P_PDB decrease from 10ms to 9ms, it is observed from Source 18 that the mean capacity performance is decreased from 8.24 UEs per cell to 8.18 UEs per cell by about 0.7%.

- For FR2, Indoor Hotspot, DL, with VR/AR Slice-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 3, with I_PER = 1% and P_PER = 1%, with I_PDB = 15ms and P_PDB decrease from 10ms to 9ms, it is observed from Source 18 that the mean capacity performance is decreased from 8.28 UEs per cell to 8.22 UEs per cell by about 0.7%.

- For FR2, Indoor Hotspot, DL, with VR/AR Slice-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 1.5, with I_PDB = 10ms and P_PDB = 10ms, with I_PER = 1% and P_PER increase from 1% to 5%, it is observed from Source 18 that the mean capacity performance is increased from 8.23 UEs per cell to 10.61 UEs per cell by about 28.9%.

- For FR2, Indoor Hotspot, DL, with VR/AR Slice-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 2, with I_PDB = 10ms and P_PDB = 10ms, with I_PER = 1% and P_PER increase from 1% to 5%, it is observed from Source 18 that the mean capacity performance is increased from 8.24 UEs per cell to 10.73 UEs per cell by about 30.2%.

- For FR2, Indoor Hotspot, DL, with VR/AR Slice-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 3, with I_PDB = 10ms and P_PDB = 10ms, with I_PER = 1% and P_PER increase from 1% to 5%, it is observed from Source 18 that the mean capacity performance is increased from 8.23 UEs per cell to 10.61 UEs per cell by about 28.9%.

- For FR2, Indoor Hotspot, DL, with VR/AR Slice-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 1.5, with I_PDB = 10ms and P_PDB = 10ms, with P_PER = 5% and I_PER decrease from 1% to 0.5%, it is observed from Source 18 that the mean capacity performance is decreased from 10.61 UEs per cell to 10.46 UEs per cell by about 1.4%.

- For FR2, Indoor Hotspot, DL, with VR/AR Slice-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 2, with I_PDB = 10ms and P_PDB = 10ms, with P_PER = 5% and I_PER decrease from 1% to 0.5%, it is observed from Source 18 that the mean capacity performance is decreased from 10.73 UEs per cell to 10.46 UEs per cell by about 2.5%.

- For FR2, Indoor Hotspot, DL, with VR/AR Slice-Based I/P Frame multi-stream traffic model, 30Mbps, 60FPS, with α = 3, with I_PDB = 10ms and P_PDB = 10ms, with P_PER = 5% and I_PER decrease from 1% to 0.5%, it is observed from Source 18 that the mean capacity performance is decreased from 10.61 UEs per cell to 10.38 UEs per cell by about 2.2%.

#### Impact of Jitter on Capacity

This section captures the capacity performance comparison for the impact of jitter on capacity, where jitter model is described as in 6.1.1.2.

Table 8.3.2.3-1. Summary for impact of jitter on Capacity

| Case | Scenario | App | PDB | R | F(fps) | MIMO | Capacity result (UEs/cell)with jitter | Capacity result (UEs/cell) without jitter | Source | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR1 DL | DU | AR/VR | 10ms | 45Mbps | 60 | SU | 5.2 | 5.4 | Source 17 |  |
|  |  |  |  | 30Mbps |  | MU | 7.15~11.5 | 7.5~11.6 | Source 9, Source 10 |  |
|  |  |  |  | 30Mbps |  | SU | 8.4 | 9 | Source 17 |  |
|  |  | CG | 15ms | 30Mbp |  | MU | 7.47 | 8.20 | Source 10 |  |
|  |  |  | 15ms | 30Mbps |  | SU | 10.2 | 10.5 | Source 17 |  |
|  |  |  | 15ms | 45Mbps |  | SU | 6.3 | 6.7 | Source 17 |  |

Table 8.3.2.3-2. Summary for impact of jitter on Capacity

| Case | Scenario | App | PDB | R(Mbps) | F(fps) | MIMO | Capacity result (UEs/cell) with jitter | Capacity result (UEs/cell) without jitter | Source | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR2 UL | DU | AR (2 streams: pose + scene) | 10ms (Pose), 30ms (Scene) | 0.2 (Pose) 10 (Scene) | 250 (Pose) 60 (Scene) | SU | 4.5 | 4.5 | Source 16 |  |
|  |  |  |  | 0.2 (Pose) 20 (Scene) |  |  | 2 | 2 | Source 16 |  |
|  | InH |  |  | 0.2 (Pose) 10 (Scene) |  |  | 5 | 5.5 | Source 16 |  |
|  |  |  |  | 0.2 (Pose) 20 (Scene) |  |  | 3.5 | 3.5 | Source 16 |  |

Based on the evaluation results in Table 8.3.2.3-1 and Table 8.3.2.3-2, the following observations can be made.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for CG traffic model, 30Mbps, 15ms PDB, 60 FPS, with SU-MIMO, it is observed from Source 17 that the mean capacity performance is increased from 10.2 UEs per cell with jitter to 10.5 UEs per cell without jitter by about 2.94%.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for CG traffic model, 45Mbps, 15ms PDB, 60 FPS, with SU-MIMO, it is observed from Source 17 that the mean capacity performance is increased from 6.3 UEs per cell with jitter to 6.7 UEs per cell without jitter by about 6.35%.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for CG traffic model, 30Mbps, 15ms PDB, 60 FPS, with MU-MIMO, it is observed from Source 10 that the mean capacity performance is increased from 7.47 UEs per cell with jitter to 8.20 UEs per cell without jitter by about 9.8%.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO, it is observed from Source 17 that the mean capacity performance is increased from 8.4 UEs per cell with jitter to 9 UEs per cell without jitter by about 7.1%.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 45Mbps, 10ms PDB, 60 FPS, with SU-MIMO, it is observed from Source 17 that the mean capacity performance is increased from 5.2 UEs per cell with jitter to 5.4 UEs per cell without jitter by about 3.85%.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with MU-MIMO, it is observed from Source 9, Source 10 that the range of capacity performance is increased from 7.15~11.5 UEs per cell with jitter to 7.5~11.6 UEs per cell without jitter and the mean capacity performance is increased from 9.33 UEs per cell with jitter to 9.55 UEs per cell without jitter by about 11.25 %.

- For FR2, Dense Urban, UL, with 100MHz bandwidth for AR 2-stream pose/control-stream and 10Mbps scene/video/ data/voice-stream, it is observed from Source 16 that the capacity performance was unchanged when jitter was introduced to the video-stream.

- For FR2, Dense Urban, UL, with 100MHz bandwidth for AR 2-stream pose/control-stream and 20Mbps scene/video/ data/voice-stream, it is observed from Source 16 that the capacity performance was unchanged when jitter was introduced to the video-stream.

- For FR2, Indoor Hotspot, UL, with 100MHz bandwidth for AR 2-stream pose/control-stream and 10Mbps scene/video/ data/voice-stream, it is observed from Source 16 that the capacity performance decreased from 5.5 to 5 when jitter was introduced to the video-stream.

- For FR2, Indoor Hotspot, UL, with 100MHz bandwidth for AR 2-stream pose/control-stream and 20Mbps scene/video/ data/voice-stream, it is observed from Source 16 that the capacity performance   was unchanged when jitter was introduced to the video-stream.

#### Impact of Dual-eye Buffers Staggering

This section captures the capacity performance comparison for the impact of dual-eye buffer, where dual-eye buffer model is described as in 6.1.1.5.

Table 8.3.2.4-1. Impact of Dual-eye Buffers Staggering

| Case | Scenario | App | PDB | R | MIMO | Capacity result (UEs/cell) |  | Source | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | F=60fps | F=120fps |  |  |
| FR1 DL | DU | AR/VR | 10ms | 45Mbps | SU | 5.77 | 8.03 | Source 18 |  |
|  |  |  |  |  | MU | 6.91 | 11.42 | Source 18 |  |
|  |  |  |  | 30Mbps | SU | 9.49 | 13.47 | Source 18 |  |
|  |  |  |  |  | MU | 13.59 | 20.78 | Source 18 |  |
|  | InH | AR/VR | 10ms | 45Mbps | SU | 4.65 | 6.59 | Source 18 |  |
|  |  |  |  |  | MU | 5.91 | 9.22 | Source 18 |  |
|  |  |  |  | 30Mbps | SU | 8.27 | 11.63 | Source 18 |  |
|  |  |  |  |  | MU | 10.8 | 16.53 | Source 18 |  |
|  | UMa | AR/VR | 10ms | 45Mbp | SU | 4.17 | 6.75 | Source 18 |  |
|  |  |  |  |  | MU | 4.68 | 8.12 | Source 18 |  |
|  |  |  |  | 30Mbp | SU | 7.24 | 11.7 | Source 18 |  |
|  |  |  |  |  | MU | 8.82 | 14.59 | Source 18 |  |
| FR2 DL | DU | AR/VR | 10ms | 30Mbps | SU | 13.44 | 16.28 | Source 18 |  |
|  |  |  |  |  | SU | 7 | 7 | Source 16 |  |
|  |  |  |  | 45Mbp | SU | 5 | 5.5 | Source 16 |  |
|  | InH | AR/VR | 10ms | 30Mbps | SU | 8.72 | 10.23 | Source 18 |  |
|  |  |  |  |  | SU | 7 | 7.5 | Source 16 |  |
|  |  |  |  | 45Mbp | SU | 5 | 5.5 | Source 16 |  |

Based on the evaluation results in Table 8.3.2.4-1, the following observations can be made.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO, it is observed from Source 18 that the mean capacity performance is increased from 9.49 UEs per cell with 60FPS to 13.47 UEs per cell with 120FPS by about 41.94%.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with MU-MIMO, it is observed from Source 18 that the mean capacity performance is increased from 13.59 UEs per cell with 60FPS to 20.78 UEs per cell with 120FPS by about 52.91%.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 45Mbps, 10ms PDB, 60 FPS, with SU-MIMO, it is observed from Source 18 that the mean capacity performance is increased from 5.77 UEs per cell with 60FPS to 8.03 UEs per cell with 120FPS by about 39.17%.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 45Mbps, 10ms PDB, 60 FPS, with MU-MIMO, it is observed from Source 18 that the mean capacity performance is increased from 6.91 UEs per cell with 60FPS to 11.42 UEs per cell with 120FPS by about 65.27%.

- For FR1, Indoor Hotspot, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO, it is observed from Source 18 that the mean capacity performance is increased from 8.27 UEs per cell with 60FPS to 11.63 UEs per cell with 120FPS by about 40.63%.

- For FR1, Indoor Hotspot, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with MU-MIMO, it is observed from Source 18 that the mean capacity performance is increased from 10.80 UEs per cell with 60FPS to 16.53 UEs per cell with 120FPS by about 53.06%.

- For FR1, Indoor Hotspot, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 45Mbps, 10ms PDB, 60 FPS, with SU-MIMO, it is observed from Source 18 that the mean capacity performance is increased from 4.65 UEs per cell with 60FPS to 6.59 UEs per cell with 120FPS by about 41.72%.

- For FR1, Indoor Hotspot, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 45Mbps, 10ms PDB, 60 FPS, with MU-MIMO, it is observed from Source 18 that the mean capacity performance is increased from 5.91 UEs per cell with 60FPS to 9.22 UEs per cell with 120FPS by about 56.01%.

- For FR1, Urban Macro, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO, it is observed from Source 18 that the mean capacity performance is increased from 7.24 UEs per cell with 60FPS to 11.7 UEs per cell with 120FPS by about 61.60%.

- For FR1, Urban Macro, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with MU-MIMO, it is observed from Source 18 that the mean capacity performance is increased from 8.82 UEs per cell with 60FPS to 14.59 UEs per cell with 120FPS by about 65.42%.

- For FR1, Urban Macro, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 45Mbps, 10ms PDB, 60 FPS, with SU-MIMO, it is observed from Source 18 that the mean capacity performance is increased from 4.17 UEs per cell with 60FPS to 6.75 UEs per cell with 120FPS by about 61.87%.

- For FR1, Urban Macro, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 45Mbps, 10ms PDB, 60 FPS, with MU-MIMO, it is observed from Source 18 that the mean capacity performance is increased from 4.68 UEs per cell with 60FPS to 8.12 UEs per cell with 120FPS by about 73.50%.

- For FR2, Dense Urban, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO, it is observed from Source 18 that the mean capacity performance is increased from 13.44 UEs per cell with 60FPS to 16.28 UEs per cell with 120FPS by about 21.13%.

- For FR2, Dense Urban, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO, it is observed from Source 16 that the capacity performance is unchanged with increase from 60FPS to 120FPS.

- For FR2, Dense Urban, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 45Mbps, 10ms PDB, 60 FPS, with SU-MIMO, it is observed from Source 18 that the mean capacity performance is increased from 8.20 UEs per cell with 60FPS to 10.32 UEs per cell with 120FPS by about 25.85%.

- For FR2, Indoor Hotspot, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO, it is observed from Source 18 that the mean capacity performance is increased from 8.72 UEs per cell with 60FPS to 10.23 UEs per cell with 120FPS by about 17.32%.

- For FR2, Indoor Hotspot, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO, it is observed from Source 16 that the capacity performances are increased from 7 with 60FPS to 7.5 with 120FPS by about 6.67%.

- For FR2, Indoor Hotspot, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 45Mbps, 10ms PDB, 60 FPS, with SU-MIMO, it is observed from Source 18 that the mean capacity performance is increased from 4.67 UEs per cell with 60FPS to 6.03 UEs per cell with 120FPS by about 29.12%.

- For FR2, Indoor Hotspot, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 45Mbps, 10ms PDB, 60 FPS, with SU-MIMO, it is observed from Source 16 that the capacity performances are increased from 5 with 60FPS to 5.5 with 120FPS by about 10%.

#### Impact of TDD Frame Format

This section captures the capacity performance comparison for the impact of TDD frame format.

Table 8.3.2.5-1. Summary for impact of TDD frame format

| Case | App | R | PDB | F(fps) | Scenario | MIMO | Capacity result (UEs/cell) with TDD format DDDSU | Capacity result (UEs/cell) with TDD format DDDUU | Source | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR1 DL | AR/VR | 30Mbps | 10ms | 60 | DU | SU | 9.7 | 7.6 | Source 8 | Note 1 |
|  |  |  |  |  |  | MU | 12.3 | 8.7 | Source 8 | Note 1 |
|  |  |  |  |  | UMa | SU | 7 | 5.4 | Source 8 | Note 1 |
|  |  |  |  |  |  | MU | 7.7 | 6.1 | Source 8 | Note 1 |
| FR2 DL | AR/VR | 30Mbps | 10ms | 60 | DU | SU | - | 4.2 | Source 7 | Note 2 |
|  |  |  |  |  |  |  | 7 | 5.5 | Source 16 | Note 2 |
|  |  |  |  |  |  |  | 30 | 21.5 | Source 16 | Note 2,3 |
|  |  |  |  |  | InH | SU | - | 4.2 | Source 7 | Note 2 |
|  |  |  |  |  |  |  | 7 | 5.5 | Source 16 | Note 2,3 |
|  |  |  |  |  |  |  | 34 | 25 | Source 16 | Note 2,3 |
|  |  | 45Mbps | 10ms | 60 | DU | SU | - | 2 | Source 7 | Note 2 |
|  |  |  |  |  |  |  | 5 | 2.5 | Source 16 | Note 2 |
|  |  |  |  |  |  |  | 22.5 | 16.5 | Source 16 | Note 2,3 |
|  |  |  |  |  | InH | SU | 5 | 2.5 | Source 16 | Note 2 |
|  |  |  |  |  |  |  | 27 | 19 | Source 16 | Note 2,3 |
|  | VR/AR Video +Audio/data | 30Mbps | 10ms (Video) 30ms (Audio) | - | InH | SU | 6 | 4 | Source 16 | Note 2 |
|  |  |  |  |  | DU | SU | 6 | 3.5 | Source 16 | Note 2 |
| FR2 UL | Pose/control | 0.2Mbps | 10ms | 250 | DU | SU | 7.5 | 18.5 | Source 16 | Note 2 |
|  |  |  |  |  | InH | SU | 7 | 19 | Source 16 | Note 2 |
|  | AR (2 streams: pose + scene) | 0.2 (Pose), 10 (Scene) | 10ms (Pose) 30ms (Scene) | 250 (Pose) 60 (Scene) | DU | SU | 1.5 | 4.5 | Source 16 | Note 2 |
|  |  |  |  |  | InH | SU | 2.5 | 5 | Source 16 | Note 2 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: UE Antenna parameters: Option 1: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) Note 3: 400MHz bandwidth |  |  |  |  |  |  |  |  |  |  |

Table 8.3.2.5-2. Summary for impact of TDD frame format

| Case | App | R | PDB | F(fps) | Scenario | MIMO | Capacity result (UEs/cell) (DDDSU TDD format) | Capacity result (UEs/cell) (Other TDD format) | Source | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR1 DL | AR/VR | 45Mbps | 10ms | 60 | DU | SU | 6 | 0 with DDDDD DDDUU (2.6GHz) | Source 14 | Note 1 |
|  |  |  |  |  |  |  | 6 | 4.2 with DSUDD SUUDD (4.9GHz) TDD format | Source 14 | Note 1 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |  |  |  |  |  |  |  |  |  |  |

Based on the evaluation results in Table 8.3.2.5-1 and Table 8.3.2.5-2, the following observations can be made.

- For FR1, Dense Urban DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO, it is observed from Source 8 that the mean capacity performance is decreased from 9.7 UEs per cell with DDDSU TDD format to 7.6 UEs per cell with DDDUU TDD format by about 21.64%.

- For FR1, Dense Urban DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with MU-MIMO, it is observed from Source 8 that the mean capacity performance is decreased from 12.3 UEs per cell with DDDSU TDD format to 8.7 UEs per cell with DDDUU TDD format by about 29.27%.

- For FR1, Dense Urban DL, with 100MHz bandwidth for VR/AR (single-stream traffic mode, 45Mbps, 60FPS, 10ms PDB), with SU-MIMO, it is observed from Source 14 that the mean capacity performance is increased from 0 UEs per cell with DDDDD DDDUU (2.6GHz) TDD format to 4.2 UEs per cell with DSUDD SUUDD (4.9GHz) TDD format.

- For FR1, Dense Urban DL, VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO, it is observed from Source 8 that the mean capacity performance is decreased from 7 UEs per cell with DDDSU TDD format to 5.4 UEs per cell with DDDUU TDD format by about 22.86%.

- For FR1, Dense Urban DL, VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with MU-MIMO, it is observed from Source 8 that the mean capacity performance is decreased from 7.7 UEs per cell with DDDSU TDD format to 6.1 UEs per cell with DDDUU TDD format by about 20.78%.

- For FR2, Dense Urban DL, with 100MHz, VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO, it is observed from Source 16 that the mean capacity performance is decreased from 7 UEs per cell with DDDSU TDD format to 5.5 UEs per cell with DDDUU TDD format by about 21.42%.

- For FR2, Dense Urban DL, with 400MHz, VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO, it is observed from Source 16 that the mean capacity performance is decreased from 30 UEs per cell with DDDSU TDD format to 21.5 UEs per cell with DDDUU TDD format by about 28.33%.

- For FR2, Dense Urban DL, with 100MHz, VR/AR single-stream traffic model, 45Mbps, 10ms PDB, 60 FPS, with SU-MIMO, it is observed from Source 16 that the mean capacity performance is decreased from 5 UEs per cell with DDDSU TDD format to 2.5 UEs per cell with DDDUU TDD format by about 50%.

- For FR2, Dense Urban DL, with 400MHz, VR/AR single-stream traffic model, 45Mbps, 10ms PDB, 60 FPS, with SU-MIMO, it is observed from Source 16 that the mean capacity performance is decreased from 22.5 UEs per cell with DDDSU TDD format to 16.5 UEs per cell with DDDUU TDD format by about 26.67%.

- For FR2, Dense Urban DL, with 100MHz, Video +Audio/data multi-stream traffic model, 30Mbps+0.756Mbps, 60 FPS, it is observed from Source 16 that the mean capacity performance is decreased from 6 UEs per cell with DDDSU TDD format to 3.5 UEs per cell with DDDUU TDD format by about 41.67%.

- For FR2, Indoor hotspot DL, with 100MHz, VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO, it is observed from Source 16 that the capacity performances are decreased from 7 with DDDSU TDD format to 5.5 with DDDUU TDD format by about 21.42%.

- For FR2 Indoor hotspot DL, with 400MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, it is observed from Source 16 that the mean capacity performance is decreased from 34 UEs per cell with DDDSU TDD format to 25 UEs per cell with DDDUU TDD format by about 26.47%.

- For FR2 Indoor hotspot DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 45Mbps, 10ms PDB, 60 FPS, it is observed from Source 16 that the mean capacity performance is decreased from 5 UEs per cell with DDDSU TDD format to 2.5 UEs per cell with DDDUU TDD format by about 50%.

- For FR2 Indoor hotspot DL, with 400MHz bandwidth for VR/AR single-stream traffic model, 45Mbps, 10ms PDB, 60 FPS, it is observed from Source 16 that the mean capacity performance is decreased from 27 UEs per cell with DDDSU TDD format to 19 UEs per cell with DDDUU TDD format by about 29.63%.

- For FR2 Indoor hotspot DL, with 100MHz bandwidth for Video +Audio/data multi-stream traffic model, 30Mbps+0.756Mbps, it is observed from Source 16 that the capacity performance is decreased from 6 UEs per cell with DDDSU TDD format to 4 UEs per cell with DDDUU TDD format by about 33.33%.

Based on the evaluation results in Table 8.3.2.5-1, the following observations can be made.

- For FR2 Dense urban UL, with 100MHz bandwidth for VR/CG pose/control traffic model, 0.2Mbps, 250FPS, 10ms PDB, it is observed from Source 16 that the mean capacity performance is increased from 7.5 UEs per cell with DDDSU TDD format to 18.5 UEs per cell with DDDUU TDD format by about 146.67%.

- For FR2 Dense urban UL, with 100MHz bandwidth for AR 2-stream pose/control-stream with 0.2Mbps data rate and scene/video/ data/voice-stream with 10Mbps data rate, it is observed from Source 16 that the mean capacity performance is increased from 1.5 UEs per cell with DDDSU TDD format to 4.5 UEs per cell with DDDUU TDD format by about 200%.

- For FR2 Indoor Hotspot UL, with 100MHz bandwidth for VR/CG pose/control-stream, 0.2Mbps, 250FPS, 10ms PDB, it is observed from Source 16 that the mean capacity performance is increased from 7 UEs per cell with DDDSU TDD format to 19 UEs per cell with DDDUU TDD format by about 171.14%.

- For FR2 Indoor Hotspot UL, with 100MHz bandwidth for AR 2-stream pose/control-stream with 0.2Mbps data rate and scene/video/ data/voice-stream with 10Mbps data rate, it is observed from Source 16 that the mean capacity performance is increased from 2.5 UEs per cell with DDDSU TDD format to 5 UEs per cell with DDDUU TDD format by about 100%.

#### Impact of Bandwidth

This section captures the capacity performance comparison for the impact of system bandwidth.

Table 8.3.2.6-1. Summary for impact of bandwidth

| Case | App | R | PDB | F(fps) | Scenario | MIMO | Capacity result (UEs/cell)with 100MHz bandwidth | Capacity result (UEs/cell) with400MHz bandwidth | Source | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR2 DL | AR/VR | 30Mbps | 10ms | 60 | DU | SU | 7 | 30 | Source 16 | Note 1 |
|  |  |  |  |  |  | SU | 5.5 | 21.5 | Source 16 | Note 2 |
|  |  |  |  |  | InH | SU | 7 | 34 | Source 16 | Note 1 |
|  |  |  |  |  |  | SU | 5.5 | 25 | Source 16 | Note 2 |
|  |  | 45Mbps | 10ms | 60 | DU | SU | 5 | 22.5 | Source 16 | Note 1 |
|  |  |  |  |  |  | SU | 2.5 | 16.5 | Source 16 | Note 2 |
|  |  |  |  |  | InH | SU | 5 | 27 | Source 16 | Note 1 |
|  |  |  |  |  |  | SU | 2.5 | 19 | Source 16 | Note 2 |
|  | CG | 8Mbps | 15ms | 60 | DU | SU | 32.5 | >45 | Source 16 | Note 1 |
|  |  |  |  |  | InH | SU | 31 | 44 | Source 16 | Note 1 |
|  |  | 30Mbps | 15ms | 60 | DU | SU | 8 | 32.5 | Source 16 | Note 1 |
|  |  |  |  |  | InH | SU | 7.5 | 36 | Source 16 | Note 1 |
| FR2 UL | VR/CG pose/control-stream | 0.2Mbps | 10ms | 250 | DU | SU | 7.5 | 8.5 | Source 16 |  |
|  |  |  |  |  | InH | SU | 7 | 7 | Source 16 |  |
|  | AR 2-stream pose/control-stream | 0.2Mbps (Pose), 10Mbps (Scene) | 10ms (Pose), 30ms (Scene) | 250 (Pose), 60 (Scene) | DU | SU | 4.5 | 7 | Source 16 |  |
|  |  |  |  |  | InH | SU | 5 | 7.5 | Source 16 |  |
| Note1：DDDSU Note2：DDDUU |  |  |  |  |  |  |  |  |  |  |

Based on the evaluation results in Table 8.3.2.6-1, the following observations can be made.

- For FR2, Dense Urban, DL, for AR/VR (30 Mbps, 10ms PDB, 60FPS), DDDSU TDD format, when bandwidth is increased from 100MHz to 400MHz, it is observed from Source 16 that the mean capacity performance is increased from 7 UEs per cell to 30 UEs per cell by about 300%.

- For FR2, Dense Urban, DL, for AR/VR (30 Mbps, 10ms PDB, 60FPS), DDDUU TDD format, when bandwidth is increased from 100MHz to 400MHz, it is observed from Source 16 that the mean capacity performance is increased from 5.5 UEs per cell to 21.5 UEs per cell by about 290.9%.

- For FR2, Indoor Hotspot, DL, for AR/VR (30 Mbps, 10ms PDB, 60FPS), DDDSU TDD format, when bandwidth is increased from 100MHz to 400MHz, it is observed from Source 16 that the mean capacity performance is increased from 7 UEs per cell to 34 UEs per cell by about 385.71%.

- For FR2, Indoor Hotspot, DL, for AR/VR (30 Mbps, 10ms PDB, 60FPS), DDDUU TDD format, when bandwidth is increased from 100MHz to 400MHz, it is observed from Source 16 that the mean capacity performance is increased from 5.5 UEs per cell to 25 UEs per cell by about 385.71%.

- For FR2, Dense Urban, DL, for AR/VR (45 Mbps, 10ms PDB, 60FPS), DDDSU TDD format, when bandwidth is increased from 100MHz to 400MHz, it is observed from Source 16 that the mean capacity performance is increased from 5 UEs per cell to 22.5 UEs per cell by about 350%.

- For FR2, Dense Urban, DL, for AR/VR (45 Mbps, 10ms PDB, 60FPS), DDDUU TDD format, when bandwidth is increased from 100MHz to 400MHz, it is observed from Source 16 that the mean capacity performance is increased from 2.5 UEs per cell to 16.5 UEs per cell by about 560%.

- For FR2, Indoor Hotspot, DL, for AR/VR (45 Mbps, 10ms PDB, 60FPS), DDDSU TDD format, when bandwidth is increased from 100MHz to 400MHz, it is observed from Source 16 that the mean capacity performance is increased from 5 UEs per cell to 27 UEs per cell by about 440%.

- For FR2, Indoor Hotspot, DL, for AR/VR (45 Mbps, 10ms PDB, 60FPS), DDDUU TDD format, when bandwidth is increased from 100MHz to 400MHz, it is observed from Source 16 that the mean capacity performance is increased from 2.5 UEs per cell to 19 UEs per cell by about 660%.

- For FR2, Dense Urban, DL, for CG (8 Mbps, 15ms PDB, 60FPS), DDDSU TDD format, when bandwidth is increased from 100MHz to 400MHz, it is observed from Source 16 that the mean capacity performance is increased from 32.5 UEs per cell to >45 UEs per cell.

- For FR2, Indoor Hotspot, DL, for CG (8 Mbps, 15ms PDB, 60FPS), DDDSU TDD format, when bandwidth is increased from 100MHz to 400MHz, it is observed from Source 16 that the mean capacity performance is increased from 31 UEs per cell to 44 UEs per cell by about 41.94%.

- For FR2, Dense Urban, DL, for CG (30 Mbps, 15ms PDB, 60FPS), DDDSU TDD format, when bandwidth is increased from 100MHz to 400MHz, it is observed from Source 16 that the mean capacity performance is increased from 8 UEs per cell to 32.5 UEs per cell by about 306.25%.

- For FR2, Indoor Hotspot, DL, for CG (30 Mbps, 15ms PDB, 60FPS), DDDSU TDD format, when bandwidth is increased from 100MHz to 400MHz, it is observed from Source 16 that the mean capacity performance is increased from 7.5 UEs per cell to 36 UEs per cell by about 380%.

- For FR2, Dense Urban, UL, for VR/CG (Pose/control-stream, 0.2Mbps data rate, 10ms PDB, 250 FPS), when bandwidth is increased from 100MHz to 400MHz, it is observed from Source 16 that the mean capacity performance is increased from 7.5 UEs per cell to 8.5 UEs per cell by about 13.33%.

- For FR2, Indoor Hotspot, UL, for VR/CG (Pose/control-stream, 0.2Mbps data rate, 10ms PDB, 250 FPS), when bandwidth is increased from 100MHz to 400MHz, it is observed from Source 16 that the mean capacity performance is unchanged.

- For FR2, Dense Urban, UL, for AR 2-stream (Pose/control-stream with 0.2Mbps data rate, 10ms PDB, 250FPS and scene/video/data/voice-stream with 10Mbps data rate, 30ms PDB, 60FPS), when bandwidth is increased from 100MHz to 400MHz, it is observed from Source 16 that the mean capacity performance is increased from 4.5 UEs per cell to 7 UEs per cell by about 55.56%.

- For FR2, Indoor Hotspot, UL, for AR 2-stream (Pose/control-stream with 0.2Mbps data rate, 10ms PDB, 250FPS and scene/video/data/voice-stream with 10Mbps data rate, 30ms PDB, 60FPS), when bandwidth is increased from 100MHz to 400MHz, it is observed from Source 16 that the mean capacity performance is increased from 5 UEs per cell to 7.5 UEs per cell by about 50%.

#### Impact of FDM/SDM and mini-slot

This section captures the capacity performance comparison for the impact of FDM/SDM or mini-slot based transmission.

Table 8.3.2.7-1. Summary for impact of FDM/SDM and mini-slot

| Case | App | R | PDB | F(fps) | Scenario | MIMO | Capacity result (UEs/cell)w/o FDM/SDM, w/ regular slot | Capacity result (UEs/cell) w/ FDM/SDM or mini-slot | Source | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR2 UL | VR/CG pose/control-stream | 0.2Mbps | 10ms | 250 | DU | SU | 7.5 | 15 | Source 16 | Note1 |
|  |  |  |  |  |  |  | 7.5 | 18.5 | Source 16 | Note2 |
|  |  |  |  |  |  |  | 7.5 | 26.5 | Source 16 | Note3 |
|  |  |  |  |  | InH | SU | 7 | 11.5 | Source 16 | Note1 |
|  |  |  |  |  |  |  | 7 | 20 | Source 16 | Note2 |
|  |  |  |  |  |  |  | 7 | 26 | Source 16 | Note3 |
| Note 1: with FDM/SDM Note 2: with mini-slot (gNB time multiplexes multiple users within a slot by allocating 7 symbols to each UE) Note 3: with combination of FDM/SDM and mini-slot (7 symbols to each UE) |  |  |  |  |  |  |  |  |  |  |

Based on the evaluation results in Table 8.3.2.7-1, the following observations can be made.

- For FR2, Dense Urban, UL, for VR/CG (Pose/control-stream, 0.2Mbps data rate, 10ms PDB, 250 FPS),

- Comparing between without and with FDM/SDM, it is observed from Source 16 that the mean capacity performance is increased from 7.5 UEs per cell to 15 UEs per cell by about 100%.

- Comparing between without and with mini-slot, it is observed from Source 16 that the mean capacity performance is increased from 7.5 UEs per cell to 18.5 UEs per cell by about 146.67%.

- Comparing between without and with FDM/SDM and mini-slot, it is observed from Source 16 that the mean capacity performance is increased from 7.5 UEs per cell to 26.5 UEs per cell by about 253.33%.

- For FR2, Indoor Hotspot, UL, for VR/CG (Pose/control-stream, 0.2Mbps data rate, 10ms PDB, 250 FPS),

- Comparing between without and with FDM/SDM, it is observed from Source 16 that the mean capacity performance is increased from 7 UEs per cell to 11.5 UEs per cell by about 64.29%.

- Comparing between without and with mini-slot, it is observed from Source 16 that the mean capacity performance is increased from 7 UEs per cell to 20 UEs per cell by about 185.71%.

- Comparing between without and with FDM/SDM and mini-slot, it is observed from Source 16 that the mean capacity performance is increased from 7 UEs per cell to 26 UEs per cell by about 271.43%.

### Potential Capacity Enhancements

There have been no RAN1 discussion on aligning the implementation details of the proposed enhancement schemes presented in this section, or aligning the evaluation methodologies to comprehensively model them. The simulation results presented in this section are primarily results from individual sources that may have certain discrepancies in the details of the proposed enhancement schemes and/or additional assumptions made for evaluation purposes.

#### Staggering of packet arrivals at gNB among UEs

This section captures the capacity performance evaluation results of staggering packet arrival time among UEs.

Compared to the case that all UEs have the same packet arrival time (Zero offset), the capacity performance is evaluated when the packet arrival times of UEs are evenly spaced within the multimedia periodicity  (Equal offset). Meanwhile, the capacity performance is also evaluated when the  packet arrival time of UEs are random (Random offset).

- Random offset: the packet arrival times of UEs are randomly generated.

- Equal offset: the Traffic arrival offset among different UEs are equal or the same, which means that he packet arrival times of UEs are evenly spaced within the multimedia periodicity, i.e. (Evenly Spaced).

- Zero offset: the interval of packet arrival among UEs are zero, i.e. all UEs have the same packet arrival time  (All synchronized).

Table 8.3.3.1-1. FR1, DL, DU, VR/AR 30Mbps, SU-MIMO

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | reciprocity-based precoding | All Sync | 10 | 7 | 7 | 90% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | reciprocity-based precoding | Random | 10 | 8.8 | 8 | 97% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | reciprocity-based precoding | Evenly Spaced | 10 | 9.1 | 9 | 93% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | reciprocity-based precoding | All Sync | 10 | 3.1 | 3 | 92% | Note 1, 3 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | reciprocity-based precoding | Random | 10 | 6.3 | 6 | 93% | Note 1, 3 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | reciprocity-based precoding | Evenly Spaced | 10 | 8.3 | 8 | 93% | Note 1, 3 |
| Source 17 | R1-2111349 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 8.4 | 8 | 95% | Note 2 |
| Source 17 | R1-2111349 | DDDSU | SU-MIMO | reciprocity-based precoding | evenly spaced | 10 | 9.2 | 9 | 91% | Note 2 |
| Source 17 | R1-2111349 | DDDSU | SU-MIMO | reciprocity-based precoding | Zero offset | 10 | 7.4 | 7 | 95% | Note 2 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: BS antenna parameters: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1:8,2) Note 3: stream packet generation rate (Fps or Hz): 30 |  |  |  |  |  |  |  |  |  |  |

Table 8.3.3.1-2. FR1, DL, DU, VR/AR 45Mbps, SU-MIMO

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | reciprocity-based precoding | All Sync | 10 | 4.5 | 4 | 98% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | reciprocity-based precoding | Random | 10 | 5.9 | 5 | 99% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | reciprocity-based precoding | Evenly Spaced | 10 | 6.1 | 6 | 92% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | reciprocity-based precoding | All Sync | 10 | 1.8 | 1 | 97% | Note 1,3 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | reciprocity-based precoding | Random | 10 | 3.6 | 3 | 95% | Note 1,3 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | reciprocity-based precoding | Evenly Spaced | 10 | 5 | 5 | 90% | Note 1,3 |
| Source 17 | R1-2111349 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 5.2 | 5 | 94% | Note 2 |
| Source 17 | R1-2111349 | DDDSU | SU-MIMO | reciprocity-based precoding | evenly spaced | 10 | 5.4 | 5 | 97% | Note 2 |
| Source 17 | R1-2111349 | DDDSU | SU-MIMO | reciprocity-based precoding | Zero offset | 10 | 4.4 | 4 | 96% | Note 2 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: BS antenna parameters: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1:8,2) Note 3: stream packet generation rate (Fps or Hz): 30 |  |  |  |  |  |  |  |  |  |  |

Table 8.3.3.1-3. FR1, DL, DU, CG 30Mbps, SU-MIMO

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 17 | R1-2111349 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 15 | 10.2 | 10 | 92% | Note 1 |
| Source 17 | R1-2111349 | DDDSU | SU-MIMO | reciprocity-based precoding | evenly spaced | 15 | 10.3 | 10 | 93% | Note 1 |
| Source 17 | R1-2111349 | DDDSU | SU-MIMO | reciprocity-based precoding | Zero offset | 15 | 10.3 | 10 | 94% | Note 1 |
| Note 1: BS antenna parameters: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1:8,2) |  |  |  |  |  |  |  |  |  |  |

Table 8.3.3.1-4. FR1, DL, DU, CG 45Mbps, SU-MIMO

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 17 | R1-2111349 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 15 | 6.3 | 6 | 94% | Note 1 |
| Source 17 | R1-2111349 | DDDSU | SU-MIMO | reciprocity-based precoding | evenly spaced | 15 | 6.3 | 6 | 94% | Note 1 |
| Source 17 | R1-2111349 | DDDSU | SU-MIMO | reciprocity-based precoding | Zero offset | 15 | 6.4 | 6 | 96% | Note 1 |
| Note 1: BS antenna parameters: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1:8,2) |  |  |  |  |  |  |  |  |  |  |

Table 8.3.3.1-5 FR2, DL, DU, VR/AR 30 Mbps, SU-MIMO

| source | Tdoc source | TDD format | Transmission scheme | Traffic arrival offset among different Ues | PDB (ms) for stream | Capacity | C1=floor (Capacity) | % of satisfied Ues when #Ues/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 16 | R1-2112720 | DDDSU | reciprocity-based precoding | All Sync | 10 | 6 | 6 | 90% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | reciprocity-based precoding | Random | 10 | 7 | 7 | 90% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | reciprocity-based precoding | All Sync | 10 | 22.5 | 22 | 91% | Note 2 |
| Source 16 | R1-2112720 | DDDSU | reciprocity-based precoding | Random | 10 | 30 | 30 | 90% | Note 2 |
| Note 1: bandwidth = 100 MHz Note 2: bandwidth = 400 MHz |  |  |  |  |  |  |  |  |  |

Table 8.3.3.1-6 FR2, DL, InH, VR/AR 30 Mbps, SU-MIMO

| source | Tdoc source | TDD format | Transmission scheme | Traffic arrival offset among different Ues | PDB (ms) for stream | Capacity | C1=floor (Capacity) | % of satisfied Ues when #Ues/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 16 | R1-2112720 | DDDSU | reciprocity-based precoding | All Sync | 10 | 6.5 | 6 | 91% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | reciprocity-based precoding | Random | 10 | 7 | 7 | 90% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | reciprocity-based precoding | All Sync | 10 | 26.5 | 26 | 92% | Note 2 |
| Source 16 | R1-2112720 | DDDSU | reciprocity-based precoding | Random | 10 | 34 | 34 | 90% | Note 2 |
| Note 1: bandwidth = 100 MHz Note 2: bandwidth = 400 MHz |  |  |  |  |  |  |  |  |  |

Based on the evaluation results in Table 8.3.3.1-1, the following observations can be made.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source 16 that the capacity performance is increased from 7 UEs per cell with zero offset across UEs to 8.8 UEs per cell with random offset across UEs by about 25.71%.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source 16 that the capacity performance is increased from 7 UEs per cell with zero offset across UEs to 9.1 UEs per cell with equal offset across UEs by about 30.00%.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 30 FPS, with SU-MIMO and 64TxRU, it is observed from Source 16 that the capacity performance is increased from 3.1 UEs per cell with zero offset across UEs to 6.3 UEs per cell with random offset across UEs by about 103.23%.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 30 FPS, with SU-MIMO and 64TxRU, it is observed from Source 16 that the capacity performance is increased from 3.1 UEs per cell with zero offset across UEs to 8.3 UEs per cell with equal offset across UEs by about 167.74%.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 32TxRU, it is observed from Source 17 that the capacity performance is increased from 7.4 UEs per cell with zero offset across UEs to 8.4 UEs per cell with random offset across UEs by about 13.51%.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 32TxRU, it is observed from Source 17 that the capacity performance is increased from 7.4 UEs per cell with zero offset across UEs to 9.2 UEs per cell with equal offset across UEs by about 24.32%.

Based on the evaluation results in Table 8.3.3.1-2, the following observations can be made.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 45Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source 16 that the capacity performance is increased from 4.5 UEs per cell with zero offset across UEs to 5.9 UEs per cell with random offset across UEs by about 31.11%.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 45Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 64TxRU, it is observed from Source 16 that the capacity performance is increased from 4.5 UEs per cell with zero offset across UEs to 6.1 UEs per cell with equal offset across UEs by about 35.56%.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 45Mbps, 10ms PDB, 30 FPS, with SU-MIMO and 64TxRU, it is observed from Source 16 that the capacity performance is increased from 1.8 UEs per cell with zero offset across UEs to 3.6 UEs per cell with random offset across UEs by about 100.00%.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 45Mbps, 10ms PDB, 30 FPS, with SU-MIMO and 64TxRU, it is observed from Source 16 that the capacity performance is increased from 1.8 UEs per cell with zero offset across UEs to 5 UEs per cell with equal offset across UEs by about 177.78%.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 45Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 32TxRU, it is observed from Source 17 that the capacity performance is increased from 4.4 UEs per cell with zero offset across UEs to 5.2 UEs per cell with random offset across UEs by about 18.18%.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 45Mbps, 10ms PDB, 60 FPS, with SU-MIMO and 32TxRU, it is observed from Source 17 that the capacity performance is increased from 4.4 UEs per cell with zero offset across UEs to 5.4 UEs per cell with equal offset across UEs by about 22.73%.

Based on the evaluation results in Table 8.3.3.1-3, the following observations can be made.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for CG traffic model, 30Mbps, 15ms PDB, 60 FPS, with SU-MIMO and 32TxRU, it is observed from Source 17 that the capacity performance is 10.3 UEs per cell with zero offset across UEs, 10.2 UEs per cell with random offset across UEs, and 10.3 UEs per cell with equal offset across UEs.

Based on the evaluation results in Table 8.3.3.1-4, the following observations can be made.

- For FR1, Dense Urban, DL, with 100MHz bandwidth for CG traffic model, 45Mbps, 15ms PDB, 60 FPS, with SU-MIMO and 32TxRU, it is observed from Source 17 that the capacity performance is 6.4 UEs per cell with zero offset across UEs, 6.3 UEs per cell with random offset across UEs, and 6.3 UEs per cell with equal offset across UEs.

Based on the evaluation results in Table 8.3.3.1-5, the following observations can be made.

- For FR2, Dense Urban, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO, it is observed from Source 16 that the capacity performances are increased from 6 with synchronized arrival offsets across UEs to 7 with random arrival offsets across UEs by about 16.67%.

- For FR2, Dense Urban, DL, with 400MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO, it is observed from Source 16 that the capacity performances are increased from 22.5 with synchronized arrival offsets across UEs to 30 with random arrival offsets across UEs by about 33.33%.

Based on the evaluation results in Table 8.3.3.1-6, the following observations can be made.

- For FR2, Indoor Hotspot, DL, with 100MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO, it is observed from Source 16 that the capacity performances are increased from 6.5 with synchronized arrival offsets across UEs to 7 with random arrival offsets across UEs by about 7.69%.

- For FR2, Indoor Hotspot, DL, with 400MHz bandwidth for VR/AR single-stream traffic model, 30Mbps, 10ms PDB, 60 FPS, with SU-MIMO, it is observed from Source 16 that the capacity performances are increased from 26.5 with synchronized arrival offsets across UEs to 34 with random arrival offsets across UEs by about 28.30%.

#### Delay Aware Scheduler

This section describes the capacity performance with Delay Aware Scheduler relative to the typical PF scheduler.

- Delay aware scheduler: during scheduling, gNB considers factors including: the remaining delivery time of the frame, etc.

The observations for capacity performance evaluation with delay aware scheduler can be summarized as follows.

- For FR1, Dense Urban, DL, with CG, 30Mbps, 60FPS, 15ms PDB, with SU-MIMO, it is observed from Source 18 that the capacity performance is increased from 11.68 UEs per cell with PF scheduler to 13.58 UEs per cell with delay-aware scheduler by about 16.27%.

- For FR1, Dense Urban, DL, with CG, 30Mbps, 60FPS, 15ms PDB, with MU-MIMO, it is observed from Source 18 that the capacity performance is increased from 19.65 UEs per cell with PF scheduler to 19.75 UEs per cell with delay-aware scheduler by about 0.51%.

- For FR1, Dense Urban, DL, with VR/AR, 30Mbps, 60FPS, 10ms PDB, with SU-MIMO, it is observed from Source 18 that the capacity performance is increased from 9.49 UEs per cell with PF scheduler to 12.67 UEs per cell with delay-aware scheduler by about 33.51%.

- For FR1, Dense Urban, DL, with VR/AR, 30Mbps, 60FPS, 10ms PDB, with MU-MIMO, it is observed from Source 18 that the capacity performance is increased from 13.59 UEs per cell with PF scheduler to 14.40 UEs per cell with delay-aware scheduler by about 5.96%.

- For FR1, Dense Urban, DL, with VR/AR I/P Frame Traffic Model, 30Mbps, 60FPS, [PDB_I, PDB_P] = [10ms, 10ms], [PER_I, PER_P] = [1%, 1%], with α = 2 and SU-MIMO, it is observed from Source 14 that the capacity performance is increased from 6 UEs per cell with PF scheduler to 8.7 UEs per cell with delay-aware scheduler by about 45%.

- For FR1, Dense Urban, DL, with VR/AR I/P Frame Traffic Model, 30Mbps, 60FPS, [PDB_I, PDB_P] = [10ms, 10ms], [PER_I, PER_P] = [0.5%, 0.5%], with α = 2 and SU-MIMO, it is observed from Source 14 that the capacity performance is increased from 6 UEs per cell with PF scheduler to 8.7 UEs per cell with delay-aware scheduler by about 45%.

- For FR1, Dense Urban, DL, with VR/AR I/P Frame Traffic Model, 30Mbps, 60FPS, [PDB_I, PDB_P] = [17ms, 9ms], [PER_I, PER_P] = [1%, 1%], with α = 2 and SU-MIMO, it is observed from Source 14 that the capacity performance is increased from 9 UEs per cell with PF scheduler to 11 UEs per cell with delay-aware scheduler by about 22.2%.

- For FR1, Dense Urban, DL, with VR/AR I/P Frame Traffic Model, 30Mbps, 60FPS, [PDB_I, PDB_P] = [10ms, 10ms], [PER_I, PER_P] = [1%, 5%], with α = 2 and SU-MIMO, it is observed from Source 14 that the capacity performance is increased from 6.5 UEs per cell with PF scheduler to 9 UEs per cell with delay-aware scheduler by about 38.5%.

- For FR1, Dense Urban, DL, with VR/AR I/P Frame Traffic Model, 30Mbps, 60FPS, [PDB_I, PDB_P] = [15ms, 10ms], [PER_I, PER_P] = [1%, 1%], with α = 2 and SU-MIMO, it is observed from Source 14 that the capacity performance is increased from 10 UEs per cell with PF scheduler to 11.5 UEs per cell with delay-aware scheduler by about 15%.

- For FR1, Dense Urban, DL, with VR/AR I/P Frame Traffic Model, 30Mbps, 60FPS, [PDB_I, PDB_P] = [15ms, 10ms], [PER_I, PER_P] = [1%, 5%], with α = 2 and SU-MIMO, it is observed from Source 14 that the capacity performance is increased from 10.3 UEs per cell with PF scheduler to 11.7 UEs per cell with delay-aware scheduler by about 13.6%.

- For FR1, Indoor Hotspot, DL, with CG, 30Mbps, 60FPS, 15ms PDB, with SU-MIMO, it is observed from Source 18 that the capacity performance is increased from 10.14 UEs per cell with PF scheduler to 11.43 UEs per cell with delay-aware scheduler by about 12.72%.

- For FR1, Indoor Hotspot, DL, with CG, 30Mbps, 60FPS, 15ms PDB, with MU-MIMO, it is observed from Source 18 that the capacity performance is increased from 16.20 UEs per cell with PF scheduler to 16.67 UEs per cell with delay-aware scheduler by about 2.90%.

- For FR1, Indoor Hotspot, DL, with VR/AR, 30Mbps, 60FPS, 10ms PDB, with SU-MIMO, it is observed from Source 18 that the capacity performance is increased from 8.27 UEs per cell with PF scheduler to 10.77 UEs per cell with delay-aware scheduler by about 30.23%.

- For FR1, Indoor Hotspot, DL, with VR/AR, 30Mbps, 60FPS, 10ms PDB, with MU-MIMO, it is observed from Source 18 that the capacity performance is increased from 10.80 UEs per cell with PF scheduler to 12.40 UEs per cell with delay-aware scheduler by about 14.81%.

- For FR1, Urban Macro, DL, with CG, 30Mbps, 60FPS, 15ms PDB, with SU-MIMO, it is observed from Source 18 that the capacity performance is increased from 10.33 UEs per cell with PF scheduler to 11.94 UEs per cell with delay-aware scheduler by about 15.59%.

- For FR1, Urban Macro, DL, with CG, 30Mbps, 60FPS, 15ms PDB, with MU-MIMO, it is observed from Source 18 that the capacity performance is increased from 14.33 UEs per cell with PF scheduler to 14.45 UEs per cell with delay-aware scheduler by about 0.84%.

- For FR1, Urban Macro, DL, with VR/AR, 30Mbps, 60FPS, 10ms PDB, with SU-MIMO, it is observed from Source 18 that the capacity performance is increased from 7.24 UEs per cell with PF scheduler to 8.56 UEs per cell with delay-aware scheduler by about 18.23%.

- For FR1, Urban Macro, DL, with VR/AR, 30Mbps, 60FPS, 10ms PDB, with MU-MIMO, it is observed from Source 18 that the capacity performance is increased from 8.82 UEs per cell with PF scheduler to 9.55 UEs per cell with delay-aware scheduler by about 8.28%.

- For FR2, Dense Urban, DL, with VR/AR, 30Mbps, 10ms PDB, with SU-MIMO, it is observed from Source 18 that the capacity performance is increased from 13.44 UEs per cell with PF scheduler to 14.16 UEs per cell with delay-aware scheduler by about 5.4%.

- For FR2, Dense Urban, DL, with VR/AR, 45Mbps, 10ms PDB, with SU-MIMO, it is observed from Source 18 that the capacity performance is increased from 8.2 UEs per cell with PF scheduler to 10.32 UEs per cell with delay-aware scheduler by about 25.9%.

- For FR2, Dense Urban, DL, with CG, 30Mbps, 15ms PDB, with SU-MIMO, it is observed from Source 18 that the capacity performance is increased from 16.16 UEs per cell with PF scheduler to 16.82 UEs per cell with delay-aware scheduler by about 4.1%.

- For FR2, Dense Urban, DL, with VR/AR two-stream (video-stream 30Mbps + audio-stream 0.756Mbps), with SU-MIMO, it is observed from Source 16 that the capacity performance is increased from 6 UEs per cell with PF scheduler to 6.5 UEs per cell with delay-aware scheduler by about 8.33%.

- For FR2, Indoor hotspot, DL, with VR/AR, 30Mbps, 10ms PDB, with SU-MIMO, it is observed from Source 18 that the capacity performance is increased from 8.72 UEs per cell with PF scheduler to 8.83 UEs per cell with delay-aware scheduler by about 1.3%.

- For FR2, Indoor hotspot, DL, with VR/AR, 45Mbps, 10ms PDB, with SU-MIMO, it is observed from Source 18 that the capacity performance is increased from 4.67 UEs per cell with PF scheduler to 6.03 UEs per cell with delay-aware scheduler by about 29.1%.

- For FR2, Indoor hotspot, DL, with CG, 30Mbps, 15ms PDB, with SU-MIMO, it is observed from Source 18 that the capacity performance is increased from 9.13 UEs per cell with PF scheduler to 10.23 UEs per cell with delay-aware scheduler by about 12.0%.

- For FR2, Indoor hotspot, DL, with VR/AR two-stream (video-stream 30Mbps + audio-stream 0.756Mbps), with SU-MIMO, it is observed from Source 16 that the capacity performance is increased from 6 UEs per cell with PF scheduler to 7 UEs per cell with delay-aware scheduler by about 16.67%.

- For FR2, Indoor hotspot, UL, with AR two-stream (Scene/video/data/audio-stream, 10Mbps, 30ms PDB, 60FPS + Pose/control-stream, 0.2Mbps, 10ms PDB, 250 FPS), with SU-MIMO, it is observed from Source 16 that the capacity performance is increased from 5 UEs per cell with PF scheduler to 6.5 UEs per cell with delay-aware scheduler by about 30.0%.

#### Frame Level Integrated Transmission Scheduler

This section describes the capacity performance with Frame Level Integrated Transmission (FLIT) Scheduler relative to the typical PF scheduler.

- FLIT scheduler: during scheduling, gNB considers factors including: the size of the frame, the size of the already sent part of the frame, the remaining delivery time of the frame, etc.

The observations for capacity performance evaluation with FLIT scheduler can be summarized as follows.

- For FR1, Dense Urban, DL, with VR/AR, 30Mbps, 60FPS, 10ms PDB, with SU-MIMO, it is observed from Source 9 that the capacity performance is increased from 5.1 UEs per cell with PF scheduler to 6.4 UEs per cell with Frame Level Integrated Transmission (FLIT) scheduler by about 25.49%.

- For FR1, Dense Urban, DL, with VR/AR, 30Mbps, 60FPS, 10ms PDB, with MU-MIMO, it is observed from Source 9 that the capacity performance is increased from 11.5 UEs per cell with PF scheduler to 14 UEs per cell with Frame Level Integrated Transmission (FLIT) scheduler by about 21.74%.

- For FR1, Dense Urban, DL, with VR/AR, 45Mbps, 60FPS, 10ms PDB, with SU-MIMO, it is observed from Source 9 that the capacity performance is increased from 2.1 UEs per cell with PF scheduler to 2.7 UEs per cell with Frame Level Integrated Transmission (FLIT) scheduler by about 28.579%.

- For FR1, Dense Urban, DL, with VR/AR, 45Mbps, 60FPS, 10ms PDB, with MU-MIMO, it is observed from Source 9 that the capacity performance is increased from 5.3 UEs per cell with PF scheduler to 6.6 UEs per cell with Frame Level Integrated Transmission (FLIT) scheduler by about 24.53%.

#### Cooperative MIMO/Precoding via Bi-directional Training (BiT)

This section captures the capacity evaluation results of bi-directional training (BiT) precoding relative to zero-forcing precoding for XR applications.

Bi-directional Training (BiT) is a spatial-domain interference avoidance scheme in a TDD Cooperative MIMO system. In BiT, DL interference probing is performed on uplink sounding resources semi-statically coordinated among gNBs. On the sounding resources, each gNB triggers SRS transmissions with parameters associated with corresponding DL transmissions. Then DL interference mitigation in spatial domain is performed by each gNB for its DL transmissions based on the interference probing outcome.

Table 8.3.3.4-1. FR1, DL, DU, VR/AR 30Mbps, SU-MIMO

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 8 | R1-2110885 | DDDUU | SU-MIMO | Zeroforcing | random | 10 | 7.6 | 7 | 92% | Note 1 |
| Source 8 | R1-2110885 | DDDUU | SU-MIMO | cooperative MIMO/precoding | random | 10 | 9.4 | 9 | 93% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | SU-MIMO | Zeroforcing | random | 10 | 9.7 | 9 | 94% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | SU-MIMO | cooperative MIMO/precoding | random | 10 | 11.7 | 11 | 92% | Note 1 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |  |  |  |  |  |  |  |  |  |  |

Table 8.3.3.4-2. FR1, DL, DU, VR/AR 30Mbps, MU-MIMO

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 8 | R1-2110885 | DDDUU | MU-MIMO | Zeroforcing | random | 10 | 8.9 | 8 | 92% | Note 1 |
| Source 8 | R1-2110885 | DDDUU | MU-MIMO | cooperative MIMO/precoding | random | 10 | 16.4 | 16 | 93% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | MU-MIMO | Zeroforcing | random | 10 | 12.3 | 12 | 92% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | MU-MIMO | cooperative MIMO/precoding | random | 10 | 20.3 | 20 | 91% | Note 1 |
| Source 8 | R1-2110885 | DDDUU | MU-MIMO | Zeroforcing | random | 7 | 6.4 | 6 | 91% | Note 1 |
| Source 8 | R1-2110885 | DDDUU | MU-MIMO | cooperative MIMO/precoding | random | 7 | 12.7 | 12 | 92% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | MU-MIMO | Zeroforcing | random | 7 | 8.4 | 8 | 92% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | MU-MIMO | cooperative MIMO/precoding | random | 7 | 16.9 | 16 | 93% | Note 1 |
| Source 8 | R1-2110885 | DDDUU | MU-MIMO | Zeroforcing | random | 13 | 11.4 | 11 | 92% | Note 1 |
| Source 8 | R1-2110885 | DDDUU | MU-MIMO | cooperative MIMO/precoding | random | 13 | 18.6 | 18 | 92% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | MU-MIMO | Zeroforcing | random | 13 | 14.7 | 14 | 91% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | MU-MIMO | cooperative MIMO/precoding | random | 13 | 22.1 | 22 | 90% | Note 1 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |  |  |  |  |  |  |  |  |  |  |

Table 8.3.3.4-3. FR1, DL, DU, VR/AR 45Mbps, SU-MIMO

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 8 | R1-2110885 | DDDUU | SU-MIMO | Zeroforcing | random | 10 | 4.0 | 4 | 90% | Note 1 |
| Source 8 | R1-2110885 | DDDUU | SU-MIMO | cooperative MIMO/precoding | random | 10 | 4.7 | 4 | 93% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | SU-MIMO | Zeroforcing | random | 10 | 6 | 6 | 90% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | SU-MIMO | cooperative MIMO/precoding | random | 10 | 7 | 7 | 90% | Note 1 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |  |  |  |  |  |  |  |  |  |  |

Table 8.3.3.4-4. FR1, DL, DU, VR/AR 45Mbps, MU-MIMO

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 8 | R1-2110885 | DDDUU | MU-MIMO | Zeroforcing | random | 10 | 5.2 | 5 | 92% | Note 1 |
| Source 8 | R1-2110885 | DDDUU | MU-MIMO | cooperative MIMO/precoding | random | 10 | 10.6 | 10 | 95% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | MU-MIMO | Zeroforcing | random | 10 | 7.3 | 7 | 94% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | MU-MIMO | cooperative MIMO/precoding | random | 10 | 14.3 | 14 | 92% | Note 1 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |  |  |  |  |  |  |  |  |  |  |

Table 8.3.3.4-5. FR1, DL, DU, CG 30Mbps, SU-MIMO

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 8 | R1-2110885 | DDDUU | SU-MIMO | Zeroforcing | random | 15 | 10.3 | 10 | 91% | Note 1 |
| Source 8 | R1-2110885 | DDDUU | SU-MIMO | cooperative MIMO/precoding | random | 15 | 11.4 | 11 | 92% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | SU-MIMO | Zeroforcing | random | 15 | 12.4 | 12 | 91% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | SU-MIMO | cooperative MIMO/precoding | random | 15 | 14.9 | 14 | 92% | Note 1 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |  |  |  |  |  |  |  |  |  |  |

Table 8.3.3.4-6. FR1, DL, DU, CG 30Mbps, MU-MIMO

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 8 | R1-2110885 | DDDUU | MU-MIMO | Zeroforcing | random | 15 | 12.3 | 12 | 92% | Note 1 |
| Source 8 | R1-2110885 | DDDUU | MU-MIMO | cooperative MIMO/precoding | random | 15 | 19.7 | 19 | 92% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | MU-MIMO | Zeroforcing | random | 15 | 17.1 | 17 | 91% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | MU-MIMO | cooperative MIMO/precoding | random | 15 | 22.9 | 22 | 91% | Note1 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |  |  |  |  |  |  |  |  |  |  |

Table 8.3.3.4-7. FR1, DL, Uma, VR/AR 30Mbps, SU-MIMO

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 8 | R1-2110885 | DDDUU | SU-MIMO | Zeroforcing | random | 10 | 5.4 | 5 | 94% | Note 1 |
| Source 8 | R1-2110885 | DDDUU | SU-MIMO | cooperative MIMO/precoding | random | 10 | 6.5 | 6 | 93% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | SU-MIMO | Zeroforcing | random | 10 | 7 | 7 | 90% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | SU-MIMO | cooperative MIMO/precoding | random | 10 | 8.8 | 8 | 92% | Note 1 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |  |  |  |  |  |  |  |  |  |  |

Table 8.3.3.4-8. FR1, DL, Uma, VR/AR 30Mbps, MU-MIMO

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 8 | R1-2110885 | DDDUU | MU-MIMO | Zeroforcing | random | 10 | 6.3 | 6 | 91% | Note 1 |
| Source 8 | R1-2110885 | DDDUU | MU-MIMO | cooperative MIMO/precoding | random | 10 | 9.5 | 9 | 92% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | MU-MIMO | Zeroforcing | random | 10 | 7.7 | 7 | 94% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | MU-MIMO | cooperative MIMO/precoding | random | 10 | 11.6 | 11 | 92% | Note 1 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |  |  |  |  |  |  |  |  |  |  |

Table 8.3.3.4-9. FR1, DL, Uma, VR/AR 45Mbps, SU-MIMO

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 8 | R1-2110885 | DDDUU | SU-MIMO | Zeroforcing | random | 10 | 3.3 | 3 | 95% | Note 1 |
| Source 8 | R1-2110885 | DDDUU | SU-MIMO | cooperative MIMO/precoding | random | 10 | 3.7 | 3 | 96% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | SU-MIMO | Zeroforcing | random | 10 | 4.4 | 4 | 93% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | SU-MIMO | cooperative MIMO/precoding | random | 10 | 5.4 | 5 | 93% | Note 1 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |  |  |  |  |  |  |  |  |  |  |

Table 8.3.3.4-10. FR1, DL, Uma, VR/AR 45Mbps, MU-MIMO

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 8 | R1-2110885 | DDDUU | MU-MIMO | Zeroforcing | random | 10 | 3.6 | 3 | 96% | Note 1 |
| Source 8 | R1-2110885 | DDDUU | MU-MIMO | cooperative MIMO/precoding | random | 10 | 5.5 | 5 | 94% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | MU-MIMO | Zeroforcing | random | 10 | 4.9 | 4 | 95% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | MU-MIMO | cooperative MIMO/precoding | random | 10 | 7.7 | 7 | 94% | Note 1 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |  |  |  |  |  |  |  |  |  |  |

Table 8.3.3.4-11. FR1, DL, Uma, CG 30Mbps, SU-MIMO

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 8 | R1-2110885 | DDDUU | SU-MIMO | Zeroforcing | random | 15 | 7.2 | 7 | 91% | Note 1 |
| Source 8 | R1-2110885 | DDDUU | SU-MIMO | cooperative MIMO/precoding | random | 15 | 8.7 | 8 | 92% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | SU-MIMO | Zeroforcing | random | 15 | 9.7 | 9 | 92% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | SU-MIMO | cooperative MIMO/precoding | random | 15 | 11.4 | 11 | 91% | Note 1 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |  |  |  |  |  |  |  |  |  |  |

Table 8.3.3.4-12. FR1, DL, Uma, CG 30Mbps, MU-MIMO

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 8 | R1-2110885 | DDDUU | MU-MIMO | Zeroforcing | random | 15 | 8.4 | 8 | 91% | Note 1 |
| Source 8 | R1-2110885 | DDDUU | MU-MIMO | cooperative MIMO/precoding | random | 15 | 12.4 | 12 | 91% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | MU-MIMO | Zeroforcing | random | 15 | 11.1 | 11 | 90% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | MU-MIMO | cooperative MIMO/precoding | random | 15 | 14.2 | 14 | 91% | Note 1 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |  |  |  |  |  |  |  |  |  |  |

Based on the evaluation results in Table 8.3.3.4-1, the following observation can be made.

- For FR1, Dense Urban, DL, for VR/AR, with single stream traffic model, DDDUU/DDDSU TDD format, with SU-MIMO, 30Mbps, 10ms PDB, 60 FPS, it is observed from Source 8 that the capacity performances are 9.4/11.7 UEs per cell with cooperative MIMO/precoding, compared to zero-forcing precoding with 7.6/9.7UEs per cell, with performance increased by 23.7%/20.6%.

Based on the evaluation results in Table 8.3.3.4-2, the following observations can be made.

- For FR1, Dense Urban, DL, for VR/AR, with single stream traffic model, DDDUU/DDDSU TDD format, with MU-MIMO, 30Mbps, 10ms PDB, 60 FPS, it is observed from Source 8 that the capacity performances are 16.4/20.3 UEs per cell with cooperative MIMO/precoding, compared to zero-forcing precoding with 8.9/12.3UEs per cell, with performance increased by 84.3%/65%.

- For FR1, Dense Urban, DL, for VR/AR, with single stream traffic model, DDDUU/DDDSU TDD format, with MU-MIMO, 30Mbps, 7ms PDB, 60 FPS, it is observed from Source 8 that the capacity performances are 12.7/16.9 UEs per cell with cooperative MIMO/precoding, compared to zero-forcing precoding with 6.4/8.4UEs per cell, with performance increased by 98%/101%.

- For FR1, Dense Urban, DL, for VR/AR, with single stream traffic model, DDDUU/DDDSU TDD format, with MU-MIMO, 30Mbps, 13ms PDB, 60 FPS, it is observed from Source 8 that the capacity performances are 18.6/22.1 UEs per cell with cooperative MIMO/precoding, compared to zero-forcing precoding with 11.4/14.7UEs per cell, with performance increased by 63%/50%.

- For FR1, Dense Urban, DL, for CG, with single stream traffic model, DDDUU/DDDSU TDD format, with SU-MIMO, 30Mbps, 15ms PDB, 60 FPS, it is observed from Source 8 that the capacity performances are 11.4/14.9 UEs per cell with cooperative MIMO/precoding, compared to zero-forcing precoding with 10.3/12.4UEs per cell, with performance increased by 10.7%/20.2%.

- For FR1, Dense Urban, DL, for CG, with single stream traffic model, DDDUU/DDDSU TDD format, with MU-MIMO, 30Mbps, 15ms PDB, 60 FPS, it is observed from Source 8 that the capacity performances are 19.7/22.9 UEs per cell with cooperative MIMO/precoding, compared to zero-forcing precoding with 12.3/17.1UEs per cell, with performance increased by 60.2%/33.9%.

Based on the evaluation results in Table 8.3.3.4-3, the following observation can be made.

- For FR1, Dense Urban, DL, for VR/AR, with single stream traffic model, DDDUU/DDDSU TDD format, with SU-MIMO, 45Mbps, 10ms PDB, 60 FPS, it is observed from Source 8 that the capacity performances are 4.7/7 UEs per cell with cooperative MIMO/precoding, compared to zero-forcing precoding with 4/6UEs per cell, with performance increased by 17.5%/16.7%.

Based on the evaluation results in Table 8.3.3.4-4, the following observation can be made.

- For FR1, Dense Urban, DL, for VR/AR, with single stream traffic model, DDDUU/DDDSU TDD format, with MU-MIMO, 45Mbps, 10ms PDB, 60 FPS, it is observed from Source 8 that the capacity performances are 10.6/14.3 UEs per cell with cooperative MIMO/precoding, compared to zero-forcing precoding with 5.2/7.3UEs per cell, with performance increased by 104%/95.9%.

Based on the evaluation results in Table 8.3.3.4-5, the following observation can be made.

- For FR1, Dense Urban, DL, for CG, with single stream traffic model, DDDUU/DDDSU TDD format, with SU-MIMO, 30Mbps, 15ms PDB, 60 FPS, it is observed from Source 8 that the capacity performances are 11.4/14.9 UEs per cell with cooperative MIMO/precoding, compared to zero-forcing precoding with 10.3/12.4UEs per cell, with performance increased by 20.4%/17.5%.

Based on the evaluation results in Table 8.3.3.4-6, the following observation can be made.

- For FR1, Dense Urban, DL, for CG, with single stream traffic model, DDDUU/DDDSU TDD format, with MU-MIMO, 30Mbps, 15ms PDB, 60 FPS, it is observed from Source 8 that the capacity performances are 19.7/22.9 UEs per cell with cooperative MIMO/precoding, compared to zero-forcing precoding with 12.3/17.1UEs per cell, with performance increased by 60.2%/33.9%.

Based on the evaluation results in Table 8.3.3.4-7, the following observation can be made.

- For FR1, Uma, DL, for VR/AR, with single stream traffic model, DDDUU/DDDSU TDD format, with SU-MIMO, 30Mbps, 10ms PDB, 60 FPS, it is observed from Source 8 that the capacity performances are 6.5/8.8 UEs per cell with cooperative MIMO/precoding, compared to zero-forcing precoding with 5.4/7UEs per cell, with performance increased by 20.4%/27%.

Based on the evaluation results in Table 8.3.3.4-8, the following observation can be made.

- For FR1, Uma, DL, for VR/AR, with single stream traffic model, DDDUU/DDDSU TDD format, with MU-MIMO, 30Mbps, 10ms PDB, 60 FPS, it is observed from Source 8 that the capacity performances are 9.5/11.6 UEs per cell with cooperative MIMO/precoding, compared to zero-forcing precoding with 6.3/7.7UEs per cell, with performance increased by 50.8%/50.6%.

Based on the evaluation results in Table 8.3.3.4-9, the following observation can be made.

- For FR1, Uma, DL, for VR/AR, with single stream traffic model, DDDUU/DDDSU TDD format, with SU-MIMO, 45Mbps, 10ms PDB, 60 FPS, it is observed from Source 8 that the capacity performances are 3.7/5.4 UEs per cell with cooperative MIMO/precoding, compared to zero-forcing precoding with 3.3/4.4UEs per cell, with performance increased by 2.1%/22.7%.

Based on the evaluation results in Table 8.3.3.4-10, the following observation can be made.

- For FR1, Uma, DL, for VR/AR, with single stream traffic model, DDDUU/DDDSU TDD format, with MU-MIMO, 45Mbps, 10ms PDB, 60 FPS, it is observed from Source 8 that the capacity performances are 5.5/7.7 UEs per cell with cooperative MIMO/precoding, compared to zero-forcing precoding with 3.6/4.9UEs per cell, with performance increased by 52.8%/57.1%.

Based on the evaluation results in Table 8.3.3.4-11, the following observation can be made.

- For FR1, Uma, DL, for CG, with single stream traffic model, DDDUU/DDDSU TDD format, with SU-MIMO, 30Mbps, 15ms PDB, 60 FPS, it is observed from Source 8 that the capacity performances are 8.7/11.4 UEs per cell with cooperative MIMO/precoding, compared to zero-forcing precoding with 7.2/9.7UEs per cell, with performance increased by 20.8%/17.5%.

Based on the evaluation results in Table 8.3.3.4-12, the following observation can be made.

- For FR1, Uma, DL, for CG, with single stream traffic model, DDDUU/DDDSU TDD format, with MU-MIMO, 30Mbps, 15ms PDB, 60 FPS, it is observed from Source 8 that the capacity performances are 12.4/14.2 UEs per cell with cooperative MIMO/precoding, compared to zero-forcing precoding with 8.4/11.1UEs per cell, with performance increased by 47.6%/27.9%.

#### Network Coding (NC)/Outer Coding (OC)

This section captures the capacity evaluation results of network/outer coding for XR applications. The network/outer coding scheme is based on introducing the NC/OC sublayer below PDCP. PDCP packets are segmented into a suitable number of sub-packets, network encoded with desirable redundancy, and handed to RLC, MAC, and PHY layer for OTA transmission. The placement for NC/OC sublayer is as a part of the RLC layer. In network/outer coding scheme provides additional redundancy reducing the overall latency of packet transmission by reducing HARQ retransmissions. In this evaluation, the baseline scheme is HARQ.

The observations for capacity performance evaluation with network/outer coding can be summarized as follows.

- For FR2, Dense urban, DL, for VR/AR, with single stream traffic model, DDDSU TDD format, with SU-MIMO, 30Mbps, 10ms PDB, network/outer coding (50% redundancy), 2CC (30&39GHz) CA, no blocking, it is observed from Source 16 that capacity performance is 8.5, compared to 8.5 UE per cell without network/outer coding.

- For FR2, Dense urban, DL, for VR/AR, with single stream traffic model, DDDSU TDD format, with SU-MIMO, 30Mbps, 10ms PDB,  network/outer coding (100% redundancy), 2CC (30&39GHz) CA, periodic blocking (4/10ms) on 30GHz CC with blocking probability 1 and channel strength attenuation of 30dB due to blocking, it is observed from Source 16 that capacity performance is 5, compared to 0 UE per cell without network/outer coding.

- For FR2, Dense urban, DL, for VR/AR, with single stream traffic model, DDDSU TDD format, with SU-MIMO, 30Mbps, 10ms PDB, network/outer coding (20% redundancy), 4CC (30,30.4,39&39.4GHz) CA, no blocking, it is observed from Source 16 that capacity performance is 15, compared to 14.5 UE per cell without network/outer coding.

- For FR2, Dense urban, DL, for VR/AR, with single stream traffic model, DDDSU TDD format, with SU-MIMO, 30Mbps, 10ms PDB,  network/outer coding (100% redundancy), 4CC (30,30.4,39&39.4GHz) CA, periodic blocking (4/10ms) on 39&39.4GHz CCs with blocking probability 1 and channel strength attenuation of 30dB due to blocking, it is observed from Source 16 that capacity performance is 9, compared to 0 UE per cell without network/outer coding.

- For FR2, Dense urban, DL, for VR/AR, with single stream traffic model, DDDSU TDD format, with SU-MIMO, 45Mbps, 10ms PDB, network/outer coding (20% redundancy), 2CC (30&39GHz) CA, no blocking, it is observed from Source 16 that capacity performance is 5, compared to 4.5 UE per cell without network/outer coding.

- For FR2, Dense urban, DL, for VR/AR, with single stream traffic model, DDDSU TDD format, with SU-MIMO, 45Mbps, 10ms PDB,  network/outer coding (100% redundancy), 2CC (30&39GHz) CA, periodic blocking (4/10ms) on 30GHz CC with blocking probability 1 and channel strength  attenuation of 30dB due to blocking, it is observed from Source 16 that capacity performance is 3, compared to 0 UE per cell without network/outer coding.

- For FR2, Dense urban, DL, for VR/AR, with single stream traffic model, DDDSU TDD format, with SU-MIMO, 45Mbps, 10ms PDB, network/outer coding (20% redundancy), 4CC (30,30.4,39&39.4GHz) CA, no blocking, it is observed from Source 16 that capacity performance is 10 compared to 10 UE per cell without network/outer coding.

- For FR2, Dense urban, DL, for VR/AR, with single stream traffic model, DDDSU TDD format, with SU-MIMO, 45Mbps, 10ms PDB, network/outer coding (100% redundancy), 4CC (30,30.4,39&39.4GHz) CA, periodic blocking (4/10ms) on 39&39.4GHz CCs with blocking probability 1 and channel strength attenuation of 30dB due to blocking, it is observed from Source 16 that capacity performance is 4, compared to 0 UE per cell without network/outer coding.

- For FR2, Dense urban, DL, for VR/AR, with single stream traffic model, DDDSU TDD format, with SU-MIMO, 45Mbps, 10ms PDB, network/outer coding (100% redundancy), mTRP (2ms evaluation interval), periodic blocking (every 40 out of 100ms with blocking probability 0.2 and channel strength attenuation of 10dB due to blocking), it is observed from (Qualcomm) that capacity performance is 9 compared to 0 UE per cell without network/outer coding.

- For FR2, Dense urban, DL, for VR/AR, with single stream traffic model, DDDSU TDD format, with SU-MIMO, 45Mbps, 10ms PDB, network/outer coding (100% redundancy), mTRP (10ms evaluation interval), periodic blocking (every 40 out of 100ms with blocking probability 0.2 and channel strength attenuation of 10dB due to blocking), it is observed from (Qualcomm) that capacity performance is 5 compared to 0 UE per cell without network/outer coding

#### gNB Scheduling Awareness UE Playout Buffer

This section captures the evaluation results of gNB Scheduling Awareness UE Playout Buffer. The XR application layer at UE would have the XR packet playout buffer to battle the delay jitter and out-of sequence XR packet arrival. The playout buffer at UE would ensure the in-sequence and time interval alignment of XR video frames when it plays out to the user. The proposed scheme is for UE to feedback not only the XR-application type (XR-application awareness) but also the implemented playout buffer at application layer to the gNB. In the evaluation, the size of playout buffer is feedback from UE and known at gNB. Then, gNB can have additional PDB, which could give gNB more time to schedule UE within the delay budget requirements of the XR service and more likely to successfully transmit packets with link adaptation gain. gNB knowing the size of playout buffer can preferentially schedule UE with packet delay close to deadline and better channel conditions.

The observations for capacity performance evaluation with gNB scheduling awareness UE playout buffer can be summarized as follows.

- For FR1, Indoor Hotspot, DL, for VR/AR, with single stream traffic model, DDDSU TDD format, with MU-MIMO, 30Mbps, 60FPS, 10ms PDB, codebook-based Type 2, it is observed from Source 3 that the capacity performance is 12 UEs per cell.

- For FR1, Indoor Hotspot, DL, for VR/AR, with single stream traffic model, DDDSU TDD format, with MU-MIMO, 30Mbps, 60FPS, 10ms PDB, codebook-based Type 2, gNB scheduling awareness of 2 frames UE playout buffer, it is observed from Source 3 that the capacity performance is 16 UEs per cell.

- For FR1, Indoor Hotspot, DL, for VR/AR, with single stream traffic model, DDDSU TDD format, with MU-MIMO, 30Mbps, 60FPS, 10ms PDB, codebook-based Type 2, gNB scheduling awareness of 3 frames UE playout buffer, it is observed from Source 3 that the capacity performance is 20 UEs per cell.

- For FR1, Indoor Hotspot, DL, for VR/AR, with single stream traffic model, DDDSU TDD format, with MU-MIMO, 30Mbps, 60FPS, 10ms PDB, codebook-based Type 2, gNB scheduling awareness of 4 frames UE playout buffer, it is observed from Source 3 that the capacity performance is 20 UEs per cell.

#### Impact of Carrier Aggregation

This section describes the capacity performance with enhanced carrier aggregation, e.g. applying CA with enhancements to a two-carrier DL CA: DDDDD DDDUU (2.6GHz) + DSUDD SUUDD (4.9GHz).

The CA enhancement here includes “cross-carrier HARQ ACK feedback” and “cross-carrier DL retransmission”.

The observations for capacity performance evaluation with enhanced carrier aggregation can be summarized as follows.

- For FR1, Dense Urban, DL, for VR/AR, with single stream traffic model, with SU-MIMO, 45Mbps, 10ms PDB, 60 FPS, it is observed from Source 14 that the capacity performance is in the range of 10.3~12.3 UEs per cell with CA with enhancements DDDDD DDDUU (2.6GHz) + DSUDD SUUDD (4.9GHz), compared with capacity performance 4.2 UEs per cell with DSUDD SUUDD (4.9GHz) or capacity performance 0 UE per cell with DSUDD SUUDD (4.9GHz).

#### Prioritizing important stream

This section describes the capacity performance with prioritizing important stream.

In the evaluation, the transmission of the more important stream, e.g. I-frame or pose/control is prioritized.

Based on the evaluation results in Table 8.3.3.8-1, the following observations can be made.

- For FR1, Dense urban, DL, with VR/AR GOP-based multi-stream traffic model, 30Mbps, 60FPS, 10ms PDB, with DDDSU, MU-MIMO, with PF scheduler, it is observed from Source 9 that the capacity performance is increased from 6 UEs per cell with no prioritization of streams to 7.4 UEs per cell with prioritizing the transmission of I frame by 23.3%.

- For FR1, Dense urban, DL, with VR/AR GOP-based multi-stream traffic model, 30Mbps, 60FPS, 10ms PDB, with DDDSU, MU-MIMO, it is observed from Source 9 that the capacity performance is increased from 6 UEs per cell with PF scheduler with no prioritization of streams to 8.6 UEs per cell with FLIT scheduler with prioritizing the transmission of I frame by 43.3%.

- For FR1, Dense urban, DL, with VR/AR GOP-based multi-stream traffic model, with [PER_I, PER_P] = [1%, 1%]/[1%, 5%]/[0.5%, 5%], 30Mbps, 60FPS, 10ms PDB, with DDDSU, MU-MIMO, with PF scheduler, it is observed from Source 18 that the capacity performances are increased from 5.2/5.2/4.74 UEs per cell with no prioritization of streams to 5.53/5.53/4.97 UEs per cell with prioritizing the transmission of I frame by 6.3%/6.3%/4.9%.

Based on the evaluation results in Table 8.3.3.8-2, the following observations can be made.

- For FR1, Dense urban, DL, with VR/AR GOP-based multi-stream traffic model with [PER_I, PER_P] = [0.5%, 5%], 45Mbps, 60FPS, 10ms PDB, with DDDSU, MU-MIMO, with PF scheduler, it is observed from Source 9 that the capacity performance is increased from 1.4 UEs per cell with no prioritization of streams to 2.6 UEs per cell with prioritizing the transmission of I frame by 85.7%.

- For FR1, Dense urban, DL, with VR/AR GOP-based multi-stream traffic model, 45Mbps, 60FPS, 10ms PDB, with DDDSU, MU-MIMO, it is observed from Source 9 that the capacity performance is increased from 1.4 UEs per cell with PF scheduler with no prioritization of streams to 3.2 UEs per cell with FLIT scheduler with prioritizing the transmission of I frame by 128.6%.

Based on the evaluation results in Table 8.3.3.8-3, the following observation can be made.

- For FR1, Dense urban, DL, with VR/AR Slice-based multi-stream traffic model, with [PER_I, PER_P] = [1%, 1%]/[1%, 5%]/[0.5%, 5%], 30Mbps, 60FPS, 10ms PDB, with DDDSU, MU-MIMO, with PF scheduler, it is observed from Source 18 that the capacity performances are 13.54/16.23/16.17 UEs per cell with prioritizing the transmission of I frame.

Based on the evaluation results in Table 8.3.3.8-4, the following observation can be made.

- For FR1, Dense urban, UL, with pose/control-stream, 0.2Mbps, 10ms PDB, 250 FPS + scene/video/ data/voice-stream, 10Mbps, 30ms PDB traffic model, with DDDSU, MU-MIMO, it is observed from Source 9 the capacity performance is increased from 1.5 UEs per cell to 5.6 UEs per cell with prioritizing the transmission of the pose/control stream by about 273.3%.

Based on the evaluation results in Table 8.3.3.8-5, the following observations can be made.

- For FR1, Indoor hotspot, DL, with coexistence between uRLLC service and XR service, with VR/AR single stream traffic model, 30Mbps, 60FPS, 10ms PDB, with DDDSU, MU-MIMO, it is observed from Source 20 that the capacity performance is increased from 8.5 UEs per cell with no preemption indication to 11.8 UEs per cell with Rel-15 Preemption by 38.8%.

- For FR1, Indoor hotspot, DL, with coexistence between uRLLC service and XR service, with VR/AR single stream traffic model, 30Mbps, 60FPS, 10ms PDB, with DDDSU, MU-MIMO, it is observed from Source 20 that the capacity performance is increased from 8.5 UEs per cell with no preemption indication to 16.6 UEs per cell with enhanced Preemption by 95.3%.

Based on the evaluation results in Table 8.3.3.8-6, the following observations can be made.

- For FR1, Indoor Hotspot, DL, with Audio/data + video multi stream traffic model, with [PER_audio, PER_video] = [0.1%, 1%], 1.12Mbps, 100FPS + 30Mbps, 60FPS, 10ms PDB, with DDDSU, MU-MIMO, with PF scheduler, it is observed from Source 20 that the capacity performance is increased from 5.7 UEs per cell with Rel-15 preemption to 8.4 UEs per cell with enhanced preemption by 47.37%.

- For FR1, Indoor Hotspot, DL, with Audio/data + video multi stream traffic model, with [PER_audio, PER_video] = [0.1%, 1%], 1.12Mbps, 100FPS + 30Mbps, 60FPS, 10ms PDB, with DDDSU, MU-MIMO, with PF scheduler, it is observed from Source 20 that the capacity performance is increased from 4.9 UEs per cell without preemption to 8.4 UEs per cell with enhanced preemption by 71.43%.

Based on the evaluation results in Table 8.3.3.8-7, the following observations can be made.

- For FR1, Indoor Hotspot, DL, with VR/AR slice-based multi stream traffic model, 30Mbps, 60FPS, 10ms PDB with [PER_I, PER_P] = [1%, 1%] and VR/AR single-stream traffic model, 30Mbps, 60FPS, 10ms, with DDDSU, MU-MIMO, with PF scheduler, it is observed from Source 20 that the capacity performance is increased from 7.1 UEs per cell without preemption to 10.2 UEs per cell with enhanced preemption by 43.66%.

- For FR1, Indoor Hotspot, DL, with VR/AR slice-based multi stream traffic model, 30Mbps, 60FPS, 10ms PDB with [PER_I, PER_P] = [1%, 1%] and VR/AR single-stream traffic model, 30Mbps, 60FPS, 10ms, with DDDSU, MU-MIMO, with PF scheduler, it is observed from Source 20 that the capacity performance is increased from 4.5 UEs per cell without preemption to 10.2 UEs per cell with enhanced preemption by 126.67%.

Table 8.3.3.8-1. FR1, DL, DU, GOP-based 30Mbps, MU-MIMO

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Traffic arrival offset among different UEs | α | [I_PDB, P_PDB] (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | random | 2 | [10,10] | 6 | 6 | 90.08% | Note 1,4 |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | random | 2 | [10,10] | 7.4 | 7 | 91.38% | Note 1,4,5 |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | random | 2 | [10,10] | 8.6 | 8 | 95.44% | Note 1,4,6 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 2 | [10,10] | 5.2 | 5 | 91.14% | Note 1,2 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 2 | [10,10] | 5.2 | 5 | 91.14% | Note 1,3 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 2 | [10,10] | 4.74 | 4 | 94.84% | Note 1,4 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 2 | [10,10] | 5.35 | 5 | 91.47% | Note 1,2,5 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 2 | [10,10] | 5.35 | 5 | 91.47% | Note 1,3,5 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 2 | [10,10] | 4.97 | 4 | 90.87% | Note 1,4,5 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: [PER_I, PER_P] = [1%, 1%] Note 3: [PER_I, PER_P] = [1%, 5%] Note 4: [PER_I, PER_P] = [0.5%, 5%] Note 5: Based on PF, prioritize the transmission of I frame Note 6: [PER_I, PER_P] = FLIT and prioritize the transmission of I frame Note 7: [PER_I, PER_P] = [10%, 1%] Note 8: [PER_I, PER_P] = [1%, 10%] |  |  |  |  |  |  |  |  |  |  |

Table 8.3.3.8-2. FR1, DL, DU, GOP-based 45Mbps, MU-MIMO

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Traffic arrival offset among different UEs | α | [I_PDB, P_PDB] (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | random | 1.5 | [10,10] | 1.4 | 1 | 97.14% | Note 1,2 |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | random | 1.5 | [10,10] | 2.6 | 2 | 92.83% | Note 1,2,3 |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | random | 1.5 | [10,10] | 3.2 | 3 | 90.79% | Note 1,2,4 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: [PER_I, PER_P] = [0.5%, 5%] Note 3: Based on PF, prioritize the transmission of I frame Note 4: [PER_I, PER_P] = FLIT and prioritize the transmission of I frame |  |  |  |  |  |  |  |  |  |  |

Table 8.3.3.8-3. FR1, DL, DU, Slice-based 30Mbps, MU-MIMO

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Traffic arrival offset among different UEs | α | [I_PDB, P_PDB] (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 2 | [10,10] | 13.69 | 13 | 92.25% | Note 1,2 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 2 | [10,10] | 16.84 | 16 | 91.77% | Note 1,3 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 2 | [10,10] | 16.59 | 16 | 91.27% | Note 1,4 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 2 | [10,10] | 13.54 | 13 | 91.72% | Note 1,2,5 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 2 | [10,10] | 16.23 | 16 | 90.77% | Note 1,3,5 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 2 | [10,10] | 16.17 | 16 | 90.57% | Note 1,4,5 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: [PER_I, PER_P] = [1%, 1%] Note 3: [PER_I, PER_P] = [1%, 5%] Note 4: [PER_I, PER_P] = [0.5%, 5%] Note 5: Based on PF, prioritize the transmission of I frame |  |  |  |  |  |  |  |  |  |  |

Table 8.3.3.8-4. FR1, UL, DU, AR (2 streams: Pose/control-stream + scene/video/data/voice-stream), 10.2Mbps, MU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | [Pose_PDB, Video_PDB] (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | Close loop rank adaptation | random | [10,30] | 1.5 | 1 | 92.38% | Note 1 |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | Close loop rank adaptation | random | [10,30] | 5.6 | 5 | 94.48% | Note 1, 2 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: Aware-traffic |  |  |  |  |  |  |  |  |  |  |

Table 8.3.3.8-5. FR1, DL, InH, VR/AR 30Mbps, MU-MIMO

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding | Random | 10 | 16.6 | 16 | 91% | Note 3, 10 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding | Random | 10 | 11.8 | 11 | 94% | Note 3, 11 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding | Random | 10 | 8.5 | 8 | 95% | Note 3, 12 |
| Note 3: 64QAM Note 10: Enhanced Preemption (XR vs. uRLLC) Note 11: Rel-15 Preemption (XR vs. uRLLC) Note 12: No Preemption (XR vs. uRLLC) |  |  |  |  |  |  |  |  |  |  |

Table 8.3.3.8-6. FR1, DL, InH, audio/data + video multi stream traffic model, MU-MIMO

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | [PDB_Audio, PDB_video] (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding | Random | [10,10] | 8.4 | 8 | 92% | Note 3, 10-1 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding | Random | [10,10] | 5.7 | 5 | 95% | Note 3, 11-1 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding | Random | [10,10] | 4.9 | 4 | 92% | Note 3, 12-1 |
| Note 3: 64QAM Note 10-1: Enhanced Preemption (Audio/data streams vs. Video streams) Note 11-1: Rel-15 Preemption(Audio/data streams vs. Video streams) Note 12-1: No Preemption (Audio/data streams vs. Video streams) |  |  |  |  |  |  |  |  |  |  |

Table 8.3.3.8-7. FR1, DL, InH, slice-based multi stream traffic model and single stream video traffic model, MU-MIMO

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | [I_PDB, P_PDB, PDB_video] (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding | Random | [10,10, 10] | 10.2 | 10 | 90% | Note 3, 10-2 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding | Random | [10,10, 10] | 7.1 | 7 | 90% | Note 3, 11-2 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding | Random | [10,10, 10] | 4.5 | 4 | 93% | Note 3, 12-2 |
| Note 3: 64QAM Note 10-2: Enhanced Preemption (I-slices vs. P-slices and video streams) Note 11-2: Rel-15 Preemption(I-slices vs. P-slices and video streams) Note 12-2: No Preemption(I-slices vs. P-slices and video streams) |  |  |  |  |  |  |  |  |  |  |

#### Adaptive Inter-UE/Intra-UE Multiplexing Techniques

This section describes the capacity performance with adaptive inter-UE/intra-UE multiplexing technique. In the evaluation, enhanced preemption mechanism with finer granularity preemption area indication is evaluated. For simulation of XR traffic and uRLLC traffic, uRLLC traffic and XR traffic are considered as the two types of traffic to be transmitted in the system, where uRLLC traffic has higher priority (HP) while XR traffic has a relatively low priority (LP).  In simulation of audio/data + video multi stream traffic model, audio/data streams and video streams are also considered as the two types of streams to be transmitted in the system, where audio/data streams have higher priority while video streams have a relatively low priority. Besides, in simulation of sliced-based multi stream traffic model and single stream video traffic model, I-slices, P-slices and video streams are considered as the three types of streams to be transmitted in the system, where I-slices streams have higher priority while video streams and P-slices have a relatively low priority.

Based on the evaluation results in Table 8.3.3.9-1, the following observations can be made.

- For FR1, Indoor hotspot, DL, with coexistence between uRLLC service and XR service, with VR/AR single-stream traffic model, 30Mbps, 60FPS, 10ms PDB, with DDDSU, MU-MIMO, it is observed from Source 20 that the capacity performance is increased from 8.5 UEs per cell with no preemption indication to 11.8 UEs per cell with Rel-15 Preemption by 38.8%.

- For FR1, Indoor hotspot, DL, with coexistence between uRLLC service and XR service, with VR/AR single-stream traffic model, 30Mbps, 60FPS, 10ms PDB, with DDDSU, MU-MIMO, it is observed from Source 20 that the capacity performance is increased from 8.5 UEs per cell with no preemption indication to 16.6 UEs per cell with enhanced Preemption by 95.3%.

Based on the evaluation results in Table 8.3.3.9-2, the following observations can be made.

- For FR1, Indoor Hotspot, DL, with Audio/data + video multi stream traffic model, with [PER_audio, PER_video] = [0.1%, 1%], 1.12Mbps, 100FPS + 30Mbps, 60FPS, 10ms PDB, with DDDSU, MU-MIMO, with PF scheduler, it is observed from Source 20 that the capacity performance is increased from 5.7 UEs per cell with Rel-15 preemption to 8.4 UEs per cell with enhanced preemption by 47.37%.

- For FR1, Indoor Hotspot, DL, with Audio/data + video multi stream traffic model, with [PER_audio, PER_video] = [0.1%, 1%], 1.12Mbps, 100FPS + 30Mbps, 60FPS, 10ms PDB, with DDDSU, MU-MIMO, with PF scheduler, it is observed from Source 20 that the capacity performance is increased from 4.9 UEs per cell without preemption to 8.4 UEs per cell with enhanced preemption by 71.43%.

Based on the evaluation results in Table 8.3.3.9-3, the following observations can be made.

- For FR1, Indoor Hotspot, DL, with VR/AR slice-based multi stream traffic model, 30Mbps, 60FPS, 10ms PDB with [PER_I, PER_P] = [1%, 1%] and VR/AR single-stream traffic model, 30Mbps, 60FPS, 10ms, with DDDSU, MU-MIMO, with PF scheduler, it is observed from Source 20 that the capacity performance is increased from 7.1 UEs per cell without preemption to 10.2 UEs per cell with enhanced preemption by 43.66%.

- For FR1, Indoor Hotspot, DL, with VR/AR slice-based multi stream traffic model, 30Mbps, 60FPS, 10ms PDB with [PER_I, PER_P] = [1%, 1%] and VR/AR single-stream traffic model, 30Mbps, 60FPS, 10ms, with DDDSU, MU-MIMO, with PF scheduler, it is observed from Source 20 that the capacity performance is increased from 4.5 UEs per cell without preemption to 10.2 UEs per cell with enhanced preemption by 126.67%.

Table 8.3.3.9-1. FR1, DL, InH, VR/AR 30Mbps, MU-MIMO

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding | Random | 10 | 16.6 | 16 | 91% | Note 3, 10 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding | Random | 10 | 11.8 | 11 | 94% | Note 3, 11 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding | Random | 10 | 8.5 | 8 | 95% | Note 3, 12 |
| Note 3: 64QAM Note 10: Enhanced Preemption (XR vs. uRLLC) Note 11: Rel-15 Preemption (XR vs. uRLLC) Note 12: No Preemption (XR vs. uRLLC) |  |  |  |  |  |  |  |  |  |  |

Table 8.3.3.9-2. FR1, DL, InH, audio/data + video multi-streams traffic model, MU-MIMO

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | [PDB_Audio, PDB_video] (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding | Random | [10,10] | 8.4 | 8 | 92% | Note 3, 10-1 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding | Random | [10,10] | 5.7 | 5 | 95% | Note 3, 11-1 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding | Random | [10,10] | 4.9 | 4 | 92% | Note 3, 12-1 |
| Note 3: 64QAM Note 10-1: Enhanced Preemption (Audio/data streams vs. Video streams) Note 11-1: Rel-15 Preemption(Audio/data streams vs. Video streams) Note 12-1: No Preemption (Audio/data streams vs. Video streams) |  |  |  |  |  |  |  |  |  |  |

Table 8.3.3.9-3. FR1, DL, InH, slice-based multi-streams traffic model and single stream video traffic model, MU-MIMO

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | [I_PDB, P_PDB, PDB_video] (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding | Random | [10,10, 10] | 10.2 | 10 | 90% | Note 3, 10-2 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding | Random | [10,10, 10] | 7.1 | 7 | 90% | Note 3, 11-2 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding | Random | [10,10, 10] | 4.5 | 4 | 93% | Note 3, 12-2 |
| Note 3: 64QAM Note 10-2: Enhanced Preemption (I-slices vs. P-slices and video streams) Note 11-2: Rel-15 Preemption(I-slices vs. P-slices and video streams) Note 12-2: No Preemption(I-slices vs. P-slices and video streams) |  |  |  |  |  |  |  |  |  |  |

#### HARQ-ACK enhancement for DG scheduling

This section describes the capacity performance with HARQ-ACK enhancement for DG scheduling. In the evaluation, soft HARQ-ACK is used, where the UE provides enhanced HARQ-ACK feedback beyond the baseline single bit ACK/NACK status in the form of a Delta MCS based on PDSCH decoding. This additional information provided by the UE based on reception of a transport block is used for gNodeB to adapt the scheduling of retransmissions.

Based on the evaluation results in Table 8.3.3.10-1, the following observation can be made.

- For FR1, Dense Urban, DL, with VR/AR, single-stream traffic model, 60Mbps, 60FPS, 10ms PDB, with DDDSU, MU-MIMO, it is observed from Source 16 that the capacity performances are increased from 0/0/0 UEs per cell with baseline HARQ-Ack (gNodeB processing delay from HARQ feedback to retransmission = 4/6/8 slots) to 4.6/2.8/2 UEs per cell with Soft HARQ-Ack with (gNodeB processing delay from HARQ feedback to retransmission = 4/6/8 slots).

Based on the evaluation results in Table 8.3.3.10-2, the following observation can be made.

- For FR1, Indoor hotspot, DL, with VR/AR, single-stream traffic model, 60Mbps, 60FPS, 10ms PDB, with DDDSU, MU-MIMO, it is observed from Source 16 that the capacity performances are increased from 0/0/0 UEs per cell with baseline HARQ-Ack (gNodeB processing delay from HARQ feedback to retransmission = 4/6/8 slots) to 2.93/2.1/1.17 UEs per cell with Soft HARQ-Ack with (gNodeB processing delay from HARQ feedback to retransmission = 4/6/8 slots).

Table 8.3.3.10-1. FR1, DL, DU, VR/AR 60Mbps, MU-MIMO

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 4.6 | 4 | 94.50% | Note 1,2 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 0 | 0 | N.A. | Note 1,3 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 2.8 | 2 | 92.90% | Note 1,4 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 0 | 0 | N.A. | Note 1,5 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 2 | 2 | 90.10% | Note 1,6 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 0 | 0 | N.A. | Note 1,7 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: Soft HARQ-Ack, gNB NACK to retx delay = 4 slots Note 3: Baseline HARQ-Ack, gNB NACK to retx delay = 4 slots Note 4: Soft HARQ-Ack, gNB NACK to retx delay = 6 slots Note 5: Baseline HARQ-Ack, gNB NACK to retx delay = 6 slots Note 6: Soft HARQ-Ack, gNB NACK to retx delay = 8 slots Note 7: Baseline HARQ-Ack, gNB NACK to retx delay = 8 slots |  |  |  |  |  |  |  |  |  |  |

Table 8.3.3.10-2. FR1, DL, InH, VR/AR 60Mbps, MU-MIMO

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 2.93 | 2 | 97.70% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 0 | 0 | N.A. | Note 2 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 2.1 | 2 | 91.25% | Note 3 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 0 | 0 | N.A. | Note 4 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 1.17 | 1 | 91.25% | Note 5 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 0 | 0 | N.A. | Note 6 |
| Note 1: Soft HARQ-Ack, gNB NACK to retx delay = 4 slots Note 2: Baseline HARQ-Ack, gNB NACK to retx delay = 4 slots Note 3: Soft HARQ-Ack, gNB NACK to retx delay = 6 slots Note 4: Baseline HARQ-Ack, gNB NACK to retx delay = 6 slots Note 5: Soft HARQ-Ack, gNB NACK to retx delay = 8 slots Note 6: Baseline HARQ-Ack, gNB NACK to retx delay = 8 slots |  |  |  |  |  |  |  |  |  |  |

#### Enhanced buffer status reporting for UL transmission

This section describes the capacity performance with Enhanced buffer status reporting for UL transmission. In the evaluation, enhancements to BSR reporting could make the network's UE buffer estimation closer to the actual UE buffer value.

Based on the evaluation results in Table 8.3.3.11-1, the following observation can be made.

- For FR1, Dense Urban, UL, AR (1 stream: Scene/video/data/voice-stream), 10Mbps, 60FPS, 30ms PDB, with DDDSU, SU-MIMO, it is observed from Source 7 that the capacity performance is increased from 7 UEs per cell with legacy BSR to 8.4 UEs per cell with enhanced BSR by 20%.

Based on the evaluation results in Table 8.3.3.11-2, the following observations can be made.

- For FR1, Dense Urban, UL AR (1 stream: Scene/video/data/voice-stream), 10Mbps, 60FPS, 30ms PDB, with DDDSU, MU-MIMO, it is observed from Source 20 that the capacity performance is increased from 9.5 UEs per cell with legacy BSR to 10.9 UEs per cell with enhanced BSR by 14.47%.

Based on the evaluation results in Table 8.3.3.11-3, the following observations can be made.

- For FR1, Dense Urban, UL, AR (1 stream: Scene/video/data/voice-stream), 20Mbps, 60FPS, 30ms PDB, with DDDSU, SU-MIMO, it is observed from Source 20 that the capacity performance is increased from 3.4 UEs per cell with legacy BSR to 5.1 UEs per cell with enhanced BSR by 50%.

Table 8.3.3.11-1. FR1, UL, DU, AR (1 stream: Scene/video/data/voice-stream), 10Mbps, SU-MIMO

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 7 | R1-2112551 | DDDUU | SU-MIMO | reciprocity-based precoding | random | 30 | 7.5 | 7 |  | Note 1 |
| Source 7 | R1-2112551 | DDDUU | SU-MIMO | reciprocity-based precoding | random | 30 | 8.4 | 8 |  | Note 1, 4 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 4: Elastic BSR |  |  |  |  |  |  |  |  |  |  |

Table 8.3.3.11-2. FR1, UL, DU, AR (1 stream: Scene/video/data/voice-stream), 10Mbps, MU-MIMO

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding | Random | 30 | 9.5 | 9 | 95% | Note 1, 2, 3 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding | Random | 30 | 10.9 | 10 | 94% | Note 1, 2, 4 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: 64QAM Note 3: legacy BSR Note 4: Enhanced BSR |  |  |  |  |  |  |  |  |  |  |

Table 8.3.3.11-3. FR1, UL, DU, AR (1 stream: Scene/video/data/voice-stream), 20Mbps, MU-MIMO

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding | Random | 30 | 3.4 | 3 | 91% | Note 1, 2, 3 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding | Random | 30 | 5.1 | 5 | 90% | Note 1, 2, 4 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: 64QAM Note 3: legacy BSR Note 4: Enhanced BSR |  |  |  |  |  |  |  |  |  |  |

#### Application Data Unit (ADU) dropping

This section describes the capacity performance with Application Data Unit (ADU) dropping. In the evaluation, for ADU dropping all PDCP packets belonging to a single ADU frame are dropped after any of them have passed the PDB limit. The performance is compared with the legacy case where PDCP packet discarding is enabled, i.e. dropping PDCP packets after they have passed the PDB limit.

Based on the evaluation results in Table 8.3.3.12-1, the following observations can be made.

- For FR1, Dense Urban, DL, with VR/AR, single-stream traffic model, 30Mbps, 60FPS, 10ms PDB, with DDDSU, MU-MIMO, it is observed from Source 7 that the capacity performance is increased from 11.2 UEs per cell without ADU dropping to 12.9 UEs per cell with ADU dropping by 15.2%.

Table 8.3.3.12-1. FR1, DL, DU, VR/AR 30Mbps, MU-MIMO

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 7 | R1-2112551 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 11.2 | 11 |  | Note 1 |
| Source 7 | R1-2112551 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 12.9 | 12 |  | Note 1, 11 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 11: ADU dropping |  |  |  |  |  |  |  |  |  |  |

# XR UE Power Consumption Evaluation

## Purpose of Study

The purpose of power study is to understand the NR UE power consumption performance for XR applications, and identify any issues and performance gaps, which could be useful for understanding i) the limitation of current NR systems in supporting XR applications and ii) the potential directions for future necessary enhancements to improve power efficiency.

## KPI

The KPI for power evaluation is the UE power consumption, which is UE specific metric. The detailed method for estimating UE power consumption is given in evaluation methodology section.

The power saving gain (PSG) is determined from A: the power consumption of a power saving scheme and B: the power consumption of baseline (AlwaysOn) case; PSG = (B-A)/B×100%.

Since UE power saving gain typically comes with the loss in capacity (i.e., more precisely, the loss in the satisfied UE ratio), it also needs to be considered jointly with power consumption/power saving gain.

## UE Power Consumption Evaluation

### Baseline Power Evaluation Results

This section includes the baseline power consumption results. PS schemes considered in this section includes AlwaysOn, R15/16/17 power saving schemes such as CDRX, cross slot scheduling and MIMO layer adaptation by BWP switching, PDCCH monitoring adaptation.

- AlwaysOn: In this scheme, UE is always available for scheduling (i.e., no DRX off period). When UE is not receiving/transmitting DL/UL data, UE is assumed to keep monitoring PDCCH.

- R15/16 CDRX: Connected mode DRX scheme is assumed. (Note that no R16 wake up signal is considered.)

- Cross slot scheduling and MIMO layer adaptation by BWP switching: R16 dynamic BWP switching across different BWP with different configuration of minimum K0 and maximum MIMO layers.

- R17 PDCCH monitoring adaptation: UE skipping PDCCH monitoring based on a dynamically indicated PDCCH skipping indication and/or search space set group switching (SSSG) indication. In this scheme, it is assumed that network will send PDCCH skipping command with skipping duration(s).

- Genie: In this scheme, UE is assumed to be in a sleep state (e.g., micro/light/deep sleep as defined in TR38.840) whenever there is neither DL data reception nor UL transmission.

#### FR1

##### DL+UL Joint Evaluation

###### DU

Table 2 Summary of FR1, DL+UL joint power evaluation results for DU

| Scen-arios | App | DL Bit rate (Mbps) | PS scheme, Note 2 | System Load | PSG (%), Note 1,3 |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Mean (%) | Range (%) |  |
| DU | VR | 30 | R15/16 CDRX | High | 3.94 | 2.24 ~ 7.0 | Source 18, Source 7, Source 16 |
|  |  |  |  | Low | 3 | 2.44 ~ 3.56 | Source 18 |
|  |  |  | R17 PDCCH monitoring adaptation | High | 19.98 |  | Source 18 |
|  |  |  |  | Low | 21.06 |  | Source 18 |
|  |  | 45 | R15/16 CDRX | High | 3.04 |  | Source 16 |
|  |  |  |  | Low |  |  |  |
|  | CG | 30 | R15/16 CDRX | High | 4.52 | 2.85~7 | Source 7, Source 16 |
|  |  |  |  | Low |  |  |  |
|  | AR (UL 1 stream) | 30 | R15/16 CDRX | High | 2.1 | 1.62 ~ 2.58 | Source 18 |
|  |  |  |  | Low | 3.09 | 2.39 ~ 3.79 | Source 18 |
|  |  |  | R17 PDCCH monitoring adaptation | High | 12.25 |  | Source 18 |
|  |  |  |  | Low | 18.26 |  | Source 18 |
|  | AR (UL 2 streams) | 30 | R15/16 CDRX | High | 2.57 | 0.79 ~ 4.29 | Source 18, Source 16 |
|  |  |  |  | Low | 1.27 | 0.91 ~ 1.63 | Source 18 |
|  |  |  | R17 PDCCH monitoring adaptation | High | 11.25 |  | Source 18 |
|  |  |  |  | Low | 12.12 |  | Source 18 |
| Note 1: PSG (Power Saving Gain) is computed for the cases only with marginal loss in % of DL+UL satisfied UE. Note 2: The CDRX configurations considered in each case could be different. The details of considered R15/16 CDRX configurations in this table are listed in the following tables. Note 3: The PSG is computed with respect to power consumption of AlwaysOn scheme. |  |  |  |  |  |  |  |

            - VR

Based on the evaluation results in Table 9.3.1.1-3, the following observations can be made.

Observations

- In FR1, DL+UL joint evaluation, DU, VR30 and high load, it is observed from Source 18, Source 16 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (10/8/4, 16/14/4, 8/6/6) provides the mean power saving gain of 3.94% in the range of 2.24 ~ 7.00% with marginal loss in DL+UL UE satisfied rate.

- In FR1, DL+UL joint evaluation, DU, VR30 and high load, it is observed from Source 18 that the R17 PDCCH monitoring adaptation scheme provides the mean power saving gain is 19.98% with marginal loss in DL+UL UE satisfied rate.

Table 9.3.1.1-3 Source specific data: FR1, DL+UL, DU, VR 30Mbps, high load

| source | data row index* | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | % of UL satisfied UE | % of DL + UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 244 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 13 | 13 |  |  | 92.43% |  |
| Source 18 | 245 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | H | 13 | 13 |  |  | 90.11% | 3.31% |
| Source 18 | 246 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | H | 13 | 13 |  |  | 91.58% | 2.24% |
| Source 18 | 250 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | H | 13 | 13 |  |  | 92.19% | 19.98% |
| Source 7 | 10 | R1-2112160 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 4 | 4 |  |  | 90.00% | 0.00% |
| Source 7 | 11 | R1-2112160 | Genie | 0 | 0 | 0 |  | H | 4 | 4 |  |  | 90.00% | 17.00% |
| Source 7 | 12 | R1-2112160 | R15/16CDRX | 4 | 3 | 0 |  | H | 4 | 4 |  |  | 80.00% | 7.00% |
| Source 16 | 5 | R1-2112720 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 11 | 11 | 95.33% | 99.74% | 95.33% | 0.00% |
| Source 16 | 6 | R1-2112720 | R15/16CDRX | 8 | 6 | 6 |  | H | 11 | 11 | 94.37% | 99.74% | 94.37% | 3.22% |
| Source 16 | 7 | R1-2112720 | R15/16CDRX | 8 | 6 | 4 |  | H | 11 | 11 | 91.00% | 50.82% | 47.53% | 7.30% |
| Source 16 | 8 | R1-2112720 | Genie | 0 | 0 | 0 |  | H | 11 | 11 | 95.33% | 99.74% | 95.33% | 18.18% |
| Source 16 | 54 | R1-2112720 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 11 | 11 | 97.14% | 100.00% | 97.14% | 0.00% |
| Source 16 | 55 | R1-2112720 | R15/16CDRX | 16 | 12 | 12 |  | H | 11 | 11 | 89.35% | 79.83% | 69.87% | 1.78% |
| Source 16 | 58 | R1-2112720 | Genie | 0 | 0 | 0 |  | H | 11 | 11 | 97.14% | 100.00% | 97.14% | 24.62% |
| *data row index N means it is the N’th row in the results sheet each company has provided. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Observations

- In FR1, DL+UL joint evaluation, DU, VR30, low load, it is observed from Source 18 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (10/8/4, 16/14/4) provides the mean power saving gain of 3% in the range of 2.44 ~ 3.56% with marginal loss in DL+UL UE satisfied rate.

- In FR1, DL+UL joint evaluation, DU, VR30, low load, it is observed from Source 18 that the R17 PDCCH monitoring adaptation scheme provides the mean power saving gain of 21.06% with marginal loss in DL+UL UE satisfied rate.

Table 4 Source specific data: FR1, DL+UL, DU, VR 30Mbps, low load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | % of UL satisfied UE | % of DL + UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 236 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | L | 7 | 13 |  |  | 100.00% |  |
| Source 18 | 237 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | L | 7 | 13 |  |  | 100.00% | 3.56% |
| Source 18 | 238 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | L | 7 | 13 |  |  | 100.00% | 2.44% |
| Source 18 | 242 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | L | 7 | 13 |  |  | 100.00% | 21.06% |

Observation

- In FR1, DL+UL joint evaluation, DU, VR45, high load, it is observed from Source 16 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (8/6/6) provides the mean power saving gain of 3.04% with marginal loss in DL+UL UE satisfied rate.

Table 5 Source specific data: FR1, DL+UL, DU, VR 45Mbps, high load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | % of UL satisfied UE | % of DL + UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 16 | 17 | R1-2112720 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 7 | 7 | 95.13% | 100.00% | 95.13% | 0.00% |
| Source 16 | 18 | R1-2112720 | R15/16CDRX | 8 | 6 | 6 |  | H | 7 | 7 | 94.29% | 100.00% | 94.29% | 3.04% |
| Source 16 | 19 | R1-2112720 | R15/16CDRX | 8 | 6 | 4 |  | H | 7 | 7 | 89.66% | 47.62% | 43.54% | 7.08% |
| Source 16 | 20 | R1-2112720 | Genie | 0 | 0 | 0 |  | H | 7 | 7 | 95.13% | 100.00% | 95.13% | 17.36% |

No results available for the case of FR1, DL+UL, DU, VR 45Mbps, low load.

            - CG

Observation

- In FR1, DL+UL joint evaluation, DU, CG30, high load, it is observed from Source 7, Source 16 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (4/3/0, 8/4/6, 8/6/6) provides the mean power saving gain is 4.52% in the range of 2.85 ~ 7% with marginal loss in DL+UL UE satisfied rate.

Table 6 Source specific data: FR1, DL+UL, DU, CG 30Mbps, high load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | % of UL satisfied UE | % of DL + UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 7 | 1 | R1-2112160 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 4 | 4 |  |  | 90.00% | 0.00% |
| Source 7 | 2 | R1-2112160 | Genie | 0 | 0 | 0 |  | H | 4 | 4 |  |  | 90.00% | 17.00% |
| Source 7 | 3 | R1-2112160 | R15/16CDRX | 4 | 3 | 0 |  | H | 4 | 4 |  |  | 89.00% | 7.00% |
| Source 16 | 29 | R1-2112720 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 15 | 15 | 91.75% | 99.87% | 91.75% | 0.00% |
| Source 16 | 30 | R1-2112720 | R15/16CDRX | 8 | 6 | 4 |  | H | 15 | 15 | 91.68% | 51.05% | 47.05% | 6.66% |
| Source 16 | 31 | R1-2112720 | R15/16CDRX | 8 | 4 | 6 |  | H | 15 | 15 | 91.62% | 99.87% | 91.62% | 3.73% |
| Source 16 | 32 | R1-2112720 | R15/16CDRX | 8 | 6 | 6 |  | H | 15 | 15 | 91.75% | 99.87% | 91.75% | 2.85% |
| Source 16 | 33 | R1-2112720 | Genie | 0 | 0 | 0 |  | H | 15 | 15 | 91.75% | 99.87% | 91.75% | 17.74% |

No results available for FR1, DL+UL, DU, CG30, low load

            - AR

              - AR with UL 1 stream

Observations

- In FR1, DL+UL joint evaluation, DU, AR30 w/ UL 1 stream, high load, it is observed from Source 18 that the R15/16CDRX with configurations of (cycle/ODT/IAT) = (10/8/4, 16/14/4) provides the mean power saving gain is 2.1% in the range of 1.62 ~ 2.58% with marginal loss in DL+UL UE satisfied rate.

- In FR1, DL+UL joint evaluation, DU, AR30 w/ UL 1 stream, high load, it is observed from Source 18 that the R17 PDCCH monitoring adaptation provides the mean power saving gain is 12.25% with marginal loss in DL+UL UE satisfied rate.

Table 7 Source specific data: FR1, DL+UL, DU, AR 30Mbps w/ UL 1 stream, high load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | % of UL satisfied UE | % of DL + UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 276 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 9 | 9 |  |  | 92.59% | - |
| Source 18 | 277 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | H | 9 | 9 |  |  | 91.89% | 2.58% |
| Source 18 | 278 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | H | 9 | 9 |  |  | 92.06% | 1.62% |
| Source 18 | 282 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | H | 9 | 9 |  |  | 92.24% | 12.25% |

Observations

- In FR1, DL+UL joint evaluation, DU, AR30 w/ UL 1 stream, low load, it is observed from Source 18 that the R15/16CDRX with configurations of (cycle/ODT/IAT) = (10/8/4, 16/14/4) provides the mean power saving gain is 3.09% in the range of 2.39 ~ 3.79% with marginal loss in DL+UL UE satisfied rate.

- In FR1, DL+UL joint evaluation, DU, AR30 w/ UL 1 stream, low load, it is observed from Source 18 that the R17 PDCCH monitoring adaptation provides the mean power saving gain is 18.26% with marginal loss in DL+UL UE satisfied rate.

Table 8 Source specific data: FR1, DL+UL, DU, AR 30Mbps  w/ UL 1 stream, low load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | % of UL satisfied UE | % of DL + UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 268 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | L | 5 | 9 |  |  | 96.51% | - |
| Source 18 | 269 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | L | 5 | 9 |  |  | 96.19% | 3.79% |
| Source 18 | 270 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | L | 5 | 9 |  |  | 96.51% | 2.39% |
| Source 18 | 274 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | L | 5 | 9 |  |  | 96.19% | 18.26% |

              - AR with UL 2 streams

Observations

- In FR1, DL+UL joint evaluation, DU, AR30 w/ UL 2 streams, high load, it is observed from Source 18, Source 16 that the R15/16CDRX with configurations of (cycle/ODT/IAT) = (10/8/4, 16/14/4, 8/4/6, 8/6/6) provides the mean power saving gain is 2.57% in the range of 0.79 ~ 4.29% with marginal loss in DL+UL UE satisfied rate.

- In FR1, DL+UL joint evaluation, DU, AR30 w/ UL 2 streams, high load, it is observed from Source 18 that the R17 PDCCH monitoring adaptation provides the mean power saving gain is 11.25% with marginal loss in DL+UL UE satisfied rate.

Table 9 Source specific data: FR1, DL+UL, DU, AR 30Mbps w/ UL 2 stream, high load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | % of UL satisfied UE | % of DL + UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 308 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 7 | 7 |  |  | 92.06% | - |
| Source 18 | 309 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | H | 7 | 7 |  |  | 91.16% | 1.51% |
| Source 18 | 310 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | H | 7 | 7 |  |  | 91.61% | 0.79% |
| Source 18 | 314 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | H | 7 | 7 |  |  | 91.61% | 11.25% |
| Source 16 | 44 | R1-2112720 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 3 | 3 | 99.80% | 94.05% | 93.85% | 0.00% |
| Source 16 | 45 | R1-2112720 | R15/16CDRX | 8 | 6 | 4 |  | H | 3 | 3 | 99.80% | 44.44% | 44.44% | 7.80% |
| Source 16 | 46 | R1-2112720 | R15/16CDRX | 8 | 4 | 6 |  | H | 3 | 3 | 99.80% | 94.44% | 94.25% | 4.29% |
| Source 16 | 47 | R1-2112720 | R15/16CDRX | 8 | 6 | 6 |  | H | 3 | 3 | 99.77% | 94.33% | 94.10% | 3.67% |

Observations

- In FR1, DL+UL joint evaluation, DU, AR30 w/ UL 2 streams, low load, it is observed from Source 18 that the R15/16CDRX with configurations of (cycle/ODT/IAT) = (10/8/4, 16/14/4) provides the mean power saving gain is 1.27% in the range of 0.91% ~ 1.63% with marginal loss in DL+UL UE satisfied rate.

- In FR1, DL+UL joint evaluation, DU, AR30 w/ UL 2 streams, low load, it is observed from Source 18 that the R17 PDCCH monitoring adaptation provides the mean power saving gain is 12.12% with marginal loss in DL+UL UE satisfied rate.

Table 10 Source specific data: FR1, DL+UL, DU, AR 30Mbps w/ UL 2 stream, low load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | % of UL satisfied UE | % of DL + UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 300 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | L | 4 | 7 |  |  | 100.00% | - |
| Source 18 | 301 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | L | 4 | 7 |  |  | 100.00% | 1.63% |
| Source 18 | 302 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | L | 4 | 7 |  |  | 100.00% | 0.91% |
| Source 18 | 306 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | L | 4 | 7 |  |  | 100.00% | 12.12% |

###### InH

Table 11 Summary of FR1, DL+UL joint power evaluation results for InH

| Scen-arios | App | DL Bit rate (Mbps) | PS scheme, Note 2 | System Load | PSG(%), Note 1,3 |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Mean (%) | Range (%) |  |
| InH | VR | 30 | R15/16 CDRX | High | 4.19 | 2.33 ~6 | Source 18, Source 16, Source 20 |
|  |  |  |  | Low | 3.18 | 2.64 ~ 3.71 | Source 18 |
|  |  |  | R17 PDCCH monitoring adaptation | High | 21.78 |  | Source 18 |
|  |  |  |  | Low | 22.35 |  | Source 18 |
|  |  | 45 | R15/16 CDRX | High | 5.78 | 2.91 ~ 7.22 | Source 16, Source 20 |
|  |  |  |  | Low |  |  |  |
|  | CG | 30 | R15/16 CDRX | High | 3.88 | 2.85 ~ 4.5 | Source 16, Source 20 |
|  |  |  |  | Low |  |  |  |
|  | AR (UL 1 stream) | 30 | R15/16 CDRX | High | 2.16 | 1.69 ~ 2.62 | Source 18 |
|  |  |  |  | Low | 3.4 | 2.59 ~ 4.2 | Source 18 |
|  |  |  | R17 PDCCH monitoring adaptation | High | 13.28 |  | Source 18 |
|  |  |  |  | Low | 21.17 |  | Source 18 |
|  | AR (UL 2 streams) | 30 | R15/16 CDRX | High | 3.72 | 0.83 ~ 8.04 | Source 18, Source 16 |
|  |  |  |  | Low | 1.42 | 1.02 ~ 1.81 | Source 18 |
|  |  |  | R17 PDCCH monitoring adaptation | High | 12.51 |  | Source 18 |
|  |  |  |  | Low | 14.47 |  | Source 18 |
| Note 1 : PSG was computed for the cases only with marginal loss in % of DL+UL satisfied UE. Note 2: The CDRX configurations considered in each case could be different. The details of considered R15/16 CDRX configurations in this table are listed in the following tables. Note 3: The PSG is computed with respect to power consumption of AlwaysOn scheme. |  |  |  |  |  |  |  |

            - VR

Observations

- In FR1, DL+UL joint evaluation, InH, VR30, high load, it is observed from Source 18, Source 20, Source 16 that the R15/16CDRX with configurations of (cycle/ODT/IAT) = (10/8/4, 16/14/4, 10/8/4, 8/6/6) provides the mean power saving gain is 4.19% in the range of 2.33 ~ 6% with marginal loss in DL+UL UE satisfied rate.

- In FR1, DL+UL joint evaluation, InH, VR30, high load, it is observed from Source 18 that the R17 PDCCH monitoring adaptation provides the mean power saving gain is 21.78% with marginal loss in DL+UL UE satisfied rate.

Table 12 Source specific data: FR1, DL+UL, InH, VR 30Mbps, high load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | % of UL satisfied UE | % of DL + UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 228 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 10 | 10 |  |  | 92.50% | - |
| Source 18 | 229 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | H | 10 | 10 |  |  | 91.25% | 3.45% |
| Source 18 | 230 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | H | 10 | 10 |  |  | 91.81% | 2.33% |
| Source 18 | 234 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | H | 10 | 10 |  |  | 91.81% | 21.78% |
| Source 20 | 1 | R1-2111351 | AlwaysOn-baseline | 0 | 0 | 0 | Note 1, 2 | H | 11 | 11 | 93.18% | 100.00% | 93.18% | 0.00% |
| Source 20 | 2 | R1-2111351 | AlwaysOn-baseline | 0 | 0 | 0 | Note 1, 3 | H | 11 | 11 | 93.18% | 100.00% | 93.18% | 0.00% |
| Source 20 | 5 | R1-2111351 | R15 CDRX | 10 | 8 | 4 | Note 1, 2 | H | 11 | 11 | 90.15% | 100.00% | 90.15% | 6.00% |
| Source 20 | 6 | R1-2111351 | R15 CDRX | 10 | 8 | 4 | Note 1, 3 | H | 11 | 11 | 90.15% | 100.00% | 90.15% | 6.00% |
| Source 16 | 9 | R1-2112720 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 9 | 9 | 92.73% | 100.00% | 92.73% | 0.00% |
| Source 16 | 10 | R1-2112720 | R15/16CDRX | 8 | 6 | 6 |  | H | 9 | 9 | 92.59% | 100.00% | 92.59% | 3.18% |
| Source 16 | 11 | R1-2112720 | R15/16CDRX | 8 | 6 | 4 |  | H | 9 | 9 | 89.29% | 49.74% | 43.92% | 7.18% |
| Source 16 | 12 | R1-2112720 | Genie | 0 | 0 | 0 |  | H | 9 | 9 | 92.73% | 100.00% | 92.73% | 20.38% |
| Note 1. DL and UL were simulated separately and collected traces are combined as a single timeline for DL+UL joint power evaluation. Note 2. Option 2(Linear interpolation in linear domain) for UL power model Note 3. Option 1(two-step Quantization) for UL power model |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Observations

- In FR1, DL+UL joint evaluation, InH, VR30, low load, it is observed from Source 18 that the R15/16CDRX with configurations of (cycle/ODT/IAT) = (10/8/4, 16/14/4) provides the mean power saving gain is 3.18% in the range of 2.64 ~ 3.71% with marginal loss in DL+UL UE satisfied rate.

- In FR1, DL+UL joint evaluation, InH, VR30, low load, it is observed from Source 18 that the R17 PDCCH monitoring adaptation provides the mean power saving gain is 22.35% with marginal loss in DL+UL UE satisfied rate.

Table 13 Source specific data: FR1, DL+UL, InH, VR 30Mbps, low load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | % of UL satisfied UE | % of DL + UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 220 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | L | 5 | 10 |  |  | 100.00% | - |
| Source 18 | 221 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | L | 5 | 10 |  |  | 100.00% | 3.71% |
| Source 18 | 222 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | L | 5 | 10 |  |  | 100.00% | 2.64% |
| Source 18 | 226 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | L | 5 | 10 |  |  | 100.00% | 22.35% |
| Source 20 | 3 | R1-2111351 | AlwaysOn-baseline | 0 | 0 | 0 | Note 1, 2 | L | 10 | 11 | 93.00% | 100.00% | 93.00% | 0.00% |
| Source 20 | 4 | R1-2111351 | AlwaysOn-baseline | 0 | 0 | 0 | Note 1, 3 | L | 10 | 11 | 93.00% | 100.00% | 93.00% | 0.00% |
| Note 1. DL and UL were simulated separately and collected traces are combined as a single timeline for DL+UL joint power evaluation. Note 2. Option 2(Linear interpolation in linear domain) for UL power model Note 3. Option 1(two-step Quantization) for UL power model |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Observation

- In FR1, DL+UL joint evaluation, InH, VR45, high load, it is observed from Source 20, Source 16 that the R15/16CDRX with configurations of (cycle/ODT/IAT) = (10/8/4, 8/6/6) provides the mean power saving gain is 5.78% in the range of 2.91% ~ 7.22% with marginal loss in DL+UL UE satisfied rate.

Table 14 Source specific data: FR1, DL+UL, InH, VR 45Mbps, high load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | % of UL satisfied UE | % of DL + UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 20 | 11 | R1-2111351 | AlwaysOn-baseline | 0 | 0 | 0 | Note 1, 2 | H | 7 | 7 | 91.00% | 100.00% | 91.00% | 0.00% |
| Source 20 | 12 | R1-2111351 | AlwaysOn-baseline | 0 | 0 | 0 | Note 1,3 | H | 7 | 7 | 91.00% | 100.00% | 91.00% | 0.00% |
| Source 20 | 13 | R1-2111351 | R15 CDRX | 10 | 8 | 4 | Note 1, 2 | H | 7 | 7 | 87.00% | 100.00% | 87.00% | 7.22% |
| Source 20 | 14 | R1-2111351 | R15 CDRX | 10 | 8 | 4 | Note 1,3 | H | 7 | 7 | 87.00% | 100.00% | 87.00% | 7.22% |
| Source 16 | 21 | R1-2112720 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 6 | 6 | 90.59% | 100.00% | 90.59% | 0.00% |
| Source 16 | 22 | R1-2112720 | R15/16CDRX | 8 | 6 | 6 |  | H | 6 | 6 | 89.82% | 100.00% | 89.82% | 2.91% |
| Source 16 | 23 | R1-2112720 | R15/16CDRX | 8 | 6 | 4 |  | H | 6 | 6 | 82.56% | 49.69% | 40.59% | 6.69% |
| Source 16 | 24 | R1-2112720 | Genie | 0 | 0 | 0 |  | H | 6 | 6 | 90.59% | 100.00% | 90.59% | 19.34% |
| Source 16 | 24 | R1-2112720 | Genie | 0 | 0 | 0 |  | H | 6 | 6 | 90.59% | 100.00% | 90.59% | 19.34% |
| Note 1. DL and UL were simulated separately and collected traces are combined as a single timeline for DL+UL joint power evaluation. Note 2. Option 2(Linear interpolation in linear domain) for UL power model Note 3. Option 1(two-step Quantization) for UL power model |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

No results available for FR1, DL+UL, InH, VR45, low load case.

            - CG

Observation

- In FR1, DL+UL joint evaluation, InH, CG30, high load, it is observed from Source 20. Source 16 that the R15/16CDRX with configurations of (cycle/ODT/IAT) = (10/8/4, 8/4/6, 8/6/6) provides the mean power saving gain is 3.88% in the range of 2.85 ~ 4.5% with marginal loss in DL+UL UE satisfied rate.

Table 15 Source specific data: FR1, DL+UL, InH, CG 30Mbps, high load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | % of UL satisfied UE | % of DL + UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 20 | 19 | R1-2111351 | AlwaysOn-baseline | 0 | 0 | 0 | Note 1, 2 | H | 12 | 12 | 96.53% | 100.00% | 96.53% | 0.00% |
| Source 20 | 20 | R1-2111351 | AlwaysOn-baseline | 0 | 0 | 0 | Note 1, 3 | H | 12 | 12 | 96.53% | 100.00% | 96.53% | 0.00% |
| Source 20 | 21 | R1-2111351 | R15 CDRX | 10 | 8 | 4 | Note 1, 2 | H | 12 | 12 | 88.88% | 100.00% | 88.88% | 4.50% |
| Source 20 | 22 | R1-2111351 | R15 CDRX | 10 | 8 | 4 | Note 1, 3 | H | 12 | 12 | 88.88% | 100.00% | 88.88% | 4.50% |
| Source 16 | 34 | R1-2112720 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 11 | 11 | 91.36% | 100.00% | 91.36% | 0.00% |
| Source 16 | 35 | R1-2112720 | R15/16CDRX | 8 | 6 | 4 |  | H | 11 | 11 | 91.67% | 49.09% | 45.15% | 6.69% |
| Source 16 | 36 | R1-2112720 | R15/16CDRX | 8 | 4 | 6 |  | H | 11 | 11 | 91.97% | 100.00% | 91.97% | 3.68% |
| Source 16 | 37 | R1-2112720 | R15/16CDRX | 8 | 6 | 6 |  | H | 11 | 11 | 91.36% | 100.00% | 91.36% | 2.85% |
| Source 16 | 38 | R1-2112720 | Genie | 0 | 0 | 0 |  | H | 11 | 11 | 91.36% | 100.00% | 91.36% | 19.70% |
| Note 1. DL and UL were simulated separately and collected traces are combined as a single timeline for DL+UL joint power evaluation. Note 2. Option 2(Linear interpolation in linear domain) for UL power model Note 3. Option 1(two-step Quantization) for UL power model |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

No results available for FR1, DL+UL, InH, CG30, low load case.

            - AR

              - AR with UL 1 stream

Observations

- In FR1, DL+UL joint evaluation, InH, AR30 w/ UL 1 stream, high load, it is observed from Source 18 that the R15/16CDRX with configurations of (cycle/ODT/IAT) = (10/8/4, 16/14/4) provides the mean power saving gain is 2.16% in the range of 1.69 ~ 2.62% with marginal loss in DL+UL UE satisfied rate.

- In FR1, DL+UL joint evaluation, InH, AR30 w/ UL 1 stream, high load, it is observed from Source 18 that the R17 PDCCH monitoring adaptation provides the mean power saving gain is 13.28% with marginal loss in DL+UL UE satisfied rate.

Table 16 Source specific data: FR1, DL+UL, InH, AR 30Mbps, UL 1 stream, high load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | % of UL satisfied UE | % of DL + UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 260 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 10 | 10 |  |  | 92.50% | - |
| Source 18 | 261 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | H | 10 | 10 |  |  | 91.67% | 2.62% |
| Source 18 | 262 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | H | 10 | 10 |  |  | 91.94% | 1.69% |
| Source 18 | 266 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | H | 10 | 10 |  |  | 91.94% | 13.28% |

Observations

- In FR1, DL+UL joint evaluation, InH, AR30 w/ UL 1 stream, low load, it is observed from Source 18 that the R15/16CDRX with configurations of (cycle/ODT/IAT) = (10/8/4, 16/14/4) provides the mean power saving gain is 3.4% in the range of 2.59 ~ 4.2% with marginal loss in DL+UL UE satisfied rate.

- In FR1, DL+UL joint evaluation, InH, AR30 w/ UL 1 stream, low load, it is observed from Source 18 that the R17 PDCCH monitoring adaptation provides the mean power saving gain is 21.17% with marginal loss in DL+UL UE satisfied rate.

Table 17 Source specific data: FR1, DL+UL, InH, AR 30Mps, UL 1 stream, low load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | % of UL satisfied UE | % of DL + UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 252 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | L | 5 | 10 |  |  | 100.00% | - |
| Source 18 | 253 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | L | 5 | 10 |  |  | 100.00% | 4.20% |
| Source 18 | 254 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | L | 5 | 10 |  |  | 100.00% | 2.59% |
| Source 18 | 258 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | L | 5 | 10 |  |  | 100.00% | 21.17% |

              - AR with UL 2 streams

Observations

- In FR1, DL+UL joint evaluation, InH, AR30 w/ UL 2 streams, high load, it is observed from Source 18, Source 16 that the R15/16CDRX with configurations of (cycle/ODT/IAT) = (10/8/4, 16/14/4, 8/4/6, 8/6/6) provides the mean power saving gain is 2.64% in the range of 0.83 ~ 4.41% with marginal loss in DL+UL UE satisfied rate.

- In FR1, DL+UL joint evaluation, InH, AR30 w/ UL 2 streams, high load, it is observed from Source 18 that the R17 PDCCH monitoring adaptation provides the mean power saving gain is 12.51% with marginal loss in DL+UL UE satisfied rate.

Table 18 Source specific data: FR1, DL+UL, InH, AR 30Mbps, UL 2 streams, high load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | % of UL satisfied UE | % of DL + UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 292 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 10 | 10 |  |  | 92.22% | - |
| Source 18 | 293 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | H | 10 | 10 |  |  | 90.83% | 1.59% |
| Source 18 | 294 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | H | 10 | 10 |  |  | 91.67% | 0.83% |
| Source 18 | 298 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | H | 10 | 10 |  |  | 91.67% | 12.51% |
| Source 16 | 49 | R1-2112720 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 3 | 3 | 99.44% | 94.44% | 93.89% | 0.00% |
| Source 16 | 50 | R1-2112720 | R15/16CDRX | 8 | 6 | 4 |  | H | 3 | 3 | 99.44% | 44.44% | 44.44% | 8.04% |
| Source 16 | 51 | R1-2112720 | R15/16CDRX | 8 | 4 | 6 |  | H | 3 | 3 | 99.72% | 94.17% | 93.89% | 4.41% |
| Source 16 | 52 | R1-2112720 | R15/16CDRX | 8 | 6 | 6 |  | H | 3 | 3 | 99.44% | 94.72% | 94.44% | 3.72% |
| Source 16 | 53 | R1-2112720 | Genie | 0 | 0 | 0 |  | H | 3 | 3 | 99.44% | 94.44% | 93.89% | 20.44% |

Observations

- In FR1, DL+UL joint evaluation, InH, AR30 w/ UL 2 streams, low load, it is observed from Source 18 that the R15/16CDRX with configurations of (cycle/ODT/IAT) = (10/8/4, 16/14/4) provides the mean power saving gain is 1.42% in the range of 1.02 ~ 1.81% with marginal loss in DL+UL UE satisfied rate.

- In FR1, DL+UL joint evaluation, InH, AR30 w/ UL 2 streams, low load, it is observed from Source 18 that the R17 PDCCH monitoring adaptation provides the mean power saving gain is 14.47% with marginal loss in DL+UL UE satisfied rate.

Table 19 Source specific data: FR1, DL+UL, InH, AR 30Mbps, UL 2 streams, low load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | % of UL satisfied UE | % of DL + UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 284 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | L | 5 | 10 |  |  | 100.00% | - |
| Source 18 | 285 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | L | 5 | 10 |  |  | 100.00% | 1.81% |
| Source 18 | 286 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | L | 5 | 10 |  |  | 100.00% | 1.02% |
| Source 18 | 290 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | L | 5 | 10 |  |  | 100.00% | 14.47% |

###### UMa

Table 20 Summary of FR1, DL+UL joint power evaluation results for UMa

| Scen-arios | App | DL Bit rate (Mbps) | PS scheme, Note 2 | System Load | PSG(%), Note 1,3 |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Mean (%) | Range (%) |  |
| UMa | VR | 30 | R15/16 CDRX | High | 3.89 |  | Source 16 |
|  |  | 45 | R15/16 CDRX | High | 3.52 |  | Source 16 |
|  | CG | 30 | R15/16 CDRX | High | 4.1 |  | Source 16 |
| Note 1 : PSG was computed for the cases only with marginal loss in % of DL+UL satisfied UE. Note 2: The CDRX configurations considered in each case could be different. The details of considered R15/16 CDRX configurations in this table are listed in the following tables. Note 3: The PSG is computed with respect to power consumption of AlwaysOn scheme. |  |  |  |  |  |  |  |

            - VR

Observation

- In FR1, DL+UL joint evaluation, UMa, VR30, high load, it is observed from Source 16 that the R15/16CDRX with configurations of (cycle/ODT/IAT) = (8/6/6) provides the mean power saving gain is 3.89% with marginal loss in DL+UL UE satisfied rate.

Table 21 Source specific data: FR1, DL+UL, UMa, VR 30Mbps, high load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | % of UL satisfied UE | % of DL + UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 16 | 13 | R1-2112720 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 4 | 4 | 93.37% | 94.22% | 93.20% | 0.00% |
| Source 16 | 14 | R1-2112720 | R15/16CDRX | 8 | 6 | 6 |  | H | 4 | 4 | 93.20% | 93.71% | 93.71% | 3.89% |
| Source 16 | 15 | R1-2112720 | R15/16CDRX | 8 | 6 | 4 |  | H | 4 | 4 | 92.86% | 50.00% | 49.66% | 8.19% |
| Source 16 | 16 | R1-2112720 | Genie | 0 | 0 | 0 |  | H | 4 | 4 | 93.37% | 94.22% | 93.20% | 8.79% |

No results available for FR1, DL+UL, UMa, VR30, low load

Observation

- In FR1, DL+UL joint evaluation, UMa, VR45, high load, it is observed from Source 16 that the R15/16CDRX with configurations of (cycle/ODT/IAT) = (8/6/6) provides the mean power saving gain is 3.52% with marginal loss in DL+UL UE satisfied rate.

Table 22 Source specific data: FR1, DL+UL, UMa, VR 45Mbps, high load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | % of UL satisfied UE | % of DL + UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 16 | 25 | R1-2112720 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 3 | 3 | 91.59% | 95.08% | 91.59% | 0.00% |
| Source 16 | 26 | R1-2112720 | R15/16CDRX | 8 | 6 | 6 |  | H | 3 | 3 | 91.59% | 94.92% | 91.59% | 3.52% |
| Source 16 | 27 | R1-2112720 | R15/16CDRX | 8 | 6 | 4 |  | H | 3 | 3 | 90.00% | 48.73% | 45.87% | 7.71% |
| Source 16 | 28 | R1-2112720 | Genie | 0 | 0 | 0 |  | H | 3 | 3 | 91.59% | 95.08% | 91.59% | 8.70% |

No results available for FR1, DL+UL, UMa, VR45, low load

            - CG

Observation

- In FR1, DL+UL joint evaluation, UMa, CG30, high load, it is observed from Source 16 that the R15/16CDRX with configurations of (cycle/ODT/IAT) = (8/4/6, 8/6/6) provides the mean power saving gain is 4.10% in the range of 3.51% ~ 4.69% with marginal loss in DL+UL UE satisfied rate.

Table 23 Source specific data: FR1, DL+UL, UMa, CG 30Mbps, high load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | % of UL satisfied UE | % of DL + UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 16 | 39 | R1-2112720 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 6 | 6 | 91.95% | 92.86% | 91.16% | 0.00% |
| Source 16 | 40 | R1-2112720 | R15/16CDRX | 8 | 6 | 4 |  | H | 6 | 6 | 92.06% | 45.58% | 44.79% | 7.72% |
| Source 16 | 41 | R1-2112720 | R15/16CDRX | 8 | 4 | 6 |  | H | 6 | 6 | 92.29% | 92.63% | 91.38% | 4.69% |
| Source 16 | 42 | R1-2112720 | R15/16CDRX | 8 | 6 | 6 |  | H | 6 | 6 | 92.40% | 92.29% | 91.16% | 3.51% |
| Source 16 | 43 | R1-2112720 | Genie | 0 | 0 | 0 |  | H | 6 | 6 | 91.95% | 92.86% | 91.16% | 9.04% |

No results available for FR1, DL+UL, UMa, CG30, low load

            - AR

No results are available.

##### DL-only Evaluation

###### DU

Table 24 Summary of FR1, DL-only power evaluation results for DU

| Scen-arios | App | DL Bit rate (Mbps) | PS scheme, Note 2 | System Load | PSG(%), Note 1,3 |  | source |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Mean (%) | Range (%) |  |
| DU | VR/AR | 30 | R15/16 CDRX | High | 8.45 | 3.03 ~ 21.0 | Source 9, Source 18, Source 15, Source 7, Source 10 |
|  |  |  |  | Low | 4.64 | 3.57 ~ 5.76 | Source 9, Source 18 |
|  |  |  | R17 PDCCH monitoring adaptation | High | 18.86 |  | Source 18 |
|  |  |  |  | Low | 22.65 |  | Source 18 |
|  |  | 45 | R15/16 CDRX | High | 4.40 | 3.1~4.61 | Source 18, Source 14, Source 15 |
|  |  |  |  | Low | 4.55 | 3.53~5.56 | Source 18 |
|  |  |  | R17 PDCCH monitoring adaptation | High | 15.69 | 12.66~18.73 | Source 18 |
|  |  |  |  | Low | 21.95 |  | Source 18 |
|  |  |  | cross-slot scheduling + MIMO layer adaptation by BWP switching | High | 9.33 |  | Source 14 |
|  |  |  | cross-slot scheduling + MIMO layer adaptation + PDCCH skipping by BWP switching | High | 9.78 |  | Source 14 |
|  | CG | 30 | R15/16 CDRX | High | 8.4 | 3.3 ~ 20 | Source 9, Source 14, Source 15, Source 7 |
|  |  |  |  | Low | 10.3 | 3.57 ~ 15.7 | Source 9, Source 10 |
|  |  |  | R17 PDCCH monitoring adaptation | High | 12.86 |  | Source 14 |
|  |  |  |  | Low |  |  |  |
|  |  |  | cross-slot scheduling + MIMO layer adaptation by BWP switching | High | 8.13 |  | Source 14 |
|  |  |  |  | Low |  |  |  |
|  |  |  | cross-slot scheduling + MIMO layer adaptation + PDCCH skipping by BWP switching | High | 8.53 |  | Source 14 |
|  |  |  |  | Low |  |  |  |
| Note 1 : PSG was computed for the cases only with marginal loss in % of DL satisfied UE. Note 2: The CDRX configurations considered in each case could be different. The details of considered R15/16 CDRX configurations in this table are listed in the following tables. Note 3: The PSG is computed with respect to power consumption of AlwaysOn scheme. |  |  |  |  |  |  |  |

            - VR/AR

Observations

- In FR1, DL only evaluation, DU, VR/AR30 and high load, it is observed from Source 9, Source 18, Source 10, Source 15, Source 7, Source 16 that the R15/16CDRX with configurations of (cycle/ODT/IAT) = (10/8/4, 16/14/4, 8/6/6, 8/4/6, 4/2/2, 10/8/2, 10/8/3) scheme provides the mean power saving gain of 8.45% in the range of 3.03 ~ 21.00% with marginal loss in DL UE satisfied rate.

- The choice of a particular R15/16 CDRX configuration (cycle, on duration, and inactivity timer) greatly affects the PS gain.

- In FR1, DL only evaluation, DU, VR/AR30 and high load, it is observed from Source 18 that the R17 PDCCH monitoring adaptation scheme provides the mean power saving gain of 18.86% with marginal loss in DL UE satisfied rate.

Table 25 Source specific data: FR1, DL-only, DU, AR/VR 30Mbps, high load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 9 | 1 | R1-2110811 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 5 | 5 | 92.00% | 0.00% |
| Source 9 | 2 | R1-2110811 | R15/16CDRX | 10 | 5 | 4 |  | H | 5 | 5 | 61.05% | 14.68% |
| Source 9 | 3 | R1-2110811 | R15/16CDRX | 10 | 8 | 4 |  | H | 5 | 5 | 88.29% | 5.53% |
| Source 9 | 4 | R1-2110811 | R15/16CDRX | 16 | 8 | 8 |  | H | 5 | 5 | 0.00% | 10.70% |
| Source 9 | 5 | R1-2110811 | R15/16CDRX | 16 | 14 | 4 |  | H | 5 | 5 | 90.67% | 3.46% |
| Source 18 | 41 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 13 | 13 | 92.43% | - |
| Source 18 | 42 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | H | 13 | 13 | 90.11% | 4.70% |
| Source 18 | 43 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | H | 13 | 13 | 91.58% | 3.03% |
| Source 18 | 47 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | H | 13 | 13 | 92.43% | 18.86% |
| Source 10 | 1 | R1-2111521 | Always On | 0 | 0 | 0 |  | H | 4 | 5 | 96.00% | 0.00% |
| Source 10 | 2 | R1-2111521 | Genie | 0 | 0 | 0 |  | H | 4 | 5 | 96.00% | 58.30% |
| Source 10 | 3 | R1-2111521 | CDRX | 8 | 6 | 6 |  | H | 4 | 5 | 94.00% | 10.80% |
| Source 10 | 4 | R1-2111521 | CDRX | 8 | 4 | 6 |  | H | 4 | 5 | 82.75% | 15.70% |
| Source 19 | 3 | R1-2112573 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 7 | 7 | 92.44% | - |
| Source 19 | 4 | R1-2112573 | Genie | 0 | 0 | 0 |  | H | 7 | 7 | 92.44% | 41.71% |
| Source 15 | 36 | R1-2112572 | R15/16CDRX | 4 | 2 | 2 |  | H | 6 | 6 | 83.00% | 21.00% |
| Source 15 | 37 | R1-2112572 | R15/16CDRX | 8 | 4 | 4 |  | H | 6 | 6 | 61.00% | 18.00% |
| Source 15 | 38 | R1-2112572 | R15/16CDRX | 16 | 8 | 8 |  | H | 6 | 6 | 0.00% | 15.80% |
| Source 15 | 39 | R1-2112572 | R15/16CDRX | 10 | 8 | 2 |  | H | 6 | 6 | 93.00% | 9.20% |
| Source 15 | 40 | R1-2112572 | R15/16CDRX | 10 | 5 | 5 |  | H | 6 | 6 | 52.00% | 17.00% |
| Source 11 | 21 | R1-2111830 | AlwaysOn - baseline | - | - | - |  | H | 3 | 3 | 100.00% | 0.00% |
| Source 11 | 22 | R1-2111830 | R15/16CDRX | 16 | 12 | 4 |  | H | 3 | 3 | 75.00% | 6.925% |
| Source 11 | 23 | R1-2111830 | R15/16CDRX | 4 | 2 | 2 |  | H | 3 | 3 | 76.00% | 16.758% |
| Source 11 | 24 | R1-2111830 | R15/16CDRX | 16 | 8 | 8 |  | H | 3 | 3 | 20.00% | 10.945% |
| Source 11 | 25 | R1-2111830 | R15/16CDRX | 10 | 8 | 2 |  | H | 3 | 3 | 88.33% | 7.221% |
| Source 7 | 14 | R1-2112160 | AlwaysOn – baseline | 0 | 0 | 0 |  | H | 4 | 4 | 90.00% | 0.00% |
| Source 7 | 15 | R1-2112160 | Genie | 0 | 0 | 0 |  | H | 4 | 4 | 90.00% | 41.00% |
| Source 7 | 16 | R1-2112160 | R15/16CDRX | 10 | 8 | 3 |  | H | 4 | 4 | 84.00% | 4.00% |
| Source 7 | 17 | R1-2112160 | R15/16CDRX | 10 | 5 | 5 |  | H | 4 | 4 | 29.00% | 8.00% |
| Source 16 | 60 | R1-2112720 | ALWAYS ON | 0 | 0 | 0 |  | H | 11 | 11 | 97.75% | 0.00% |

Observations

- In FR1, DL only evaluation, DU, VR/AR30 and low load, it is observed from Source 9, Source 18 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (10/8/4, 16/14/4, 10/8/4, 16/14/4) provides the mean power saving gain of 4.64% in the range of 3.57 ~ 5.76% with marginal loss in DL UE satisfied rate.

- In FR1, DL only evaluation, DU, VR/AR30 and low load, it is observed from Source 18 that the R17 PDCCH monitoring adaptation scheme provides the mean power saving gain of 22.65% with marginal loss in DL UE satisfied rate.

Table 26 Source specific data: FR1, DL-only, DU, AR/VR 30Mbps, low load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 9 | 16 | R1-2110811 | AlwaysOn - baseline | 0 | 0 | 0 |  | L | 3 | 5 | 98.41% | 0.00% |
| Source 9 | 17 | R1-2110811 | R15/16CDRX | 10 | 5 | 4 |  | L | 3 | 5 | 78.25% | 15.24% |
| Source 9 | 18 | R1-2110811 | R15/16CDRX | 10 | 8 | 4 |  | L | 3 | 5 | 97.78% | 5.76% |
| Source 9 | 19 | R1-2110811 | R15/16CDRX | 16 | 8 | 8 |  | L | 3 | 5 | 0.00% | 11.01% |
| Source 9 | 20 | R1-2110811 | R15/16CDRX | 16 | 14 | 4 |  | L | 3 | 5 | 97.94% | 3.57% |
| Source 18 | 33 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | L | 7 | 13 | 100.00% | - |
| Source 18 | 34 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | L | 7 | 13 | 100.00% | 5.57% |
| Source 18 | 35 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | L | 7 | 13 | 100.00% | 3.65% |
| Source 18 | 39 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | L | 7 | 13 | 100.00% | 22.65% |
| Source 19 | 1 | R1-2112573 | AlwaysOn - baseline | 0 | 0 | 0 |  | L | 4 | 7 | 99.50% | - |
| Source 19 | 2 | R1-2112573 | Genie | 0 | 0 | 0 |  | L | 4 | 7 | 99.50% | 44.24% |

Observations

- In FR1, DL only evaluation, DU, VR/AR45 and high load, it is observed from Source 18, Source 15, Source 11 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (10/8/4, 16/14/4, 10/8/2) provides the mean power saving gain of 4.40% in the range of 3.10 ~ 6.61% with marginal loss in DL UE satisfied rate.

- In FR1, DL only evaluation, DU, VR/AR45 and high load, it is observed from Source 18, Source 14 that the R17 PDCCH monitoring adaptation scheme provides the mean power saving gain of 15.69% in the range of 12.66~18.73% with marginal loss in DL UE satisfied rate.

- In FR1, DL only evaluation, DU, VR/AR45 and high load, it is observed from Source 14 that the cross-slot scheduling + MIMO layer adaptation by BWP switching provides the mean power saving gain of 9.33% with marginal loss in DL UE satisfied rate.

- In FR1, DL only evaluation, DU, VR/AR45 and high load, it is observed from Source 14 that the cross-slot scheduling + MIMO layer adaptation + PDCCH skipping by BWP switching provides the mean power saving gain of 9.78% with marginal loss in DL UE satisfied rate.

Table 27 Source specific data: FR1, DL-only, DU, AR/VR 45Mbps, high load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 57 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 6 | 6 | 95.63% | - |
| Source 18 | 58 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | H | 6 | 6 | 93.12% | 4.69% |
| Source 18 | 59 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | H | 6 | 6 | 94.18% | 3.10% |
| Source 18 | 63 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | H | 6 | 6 | 94.44% | 18.73% |
| Source 20 | 40 | R1-2111351 | AlwaysOn-baseline | 0 | 0 | 0 |  | H | 7 | 7 | 96.60% | 0.00% |
| Source 19 | 11 | R1-2112573 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 5 | 5 | 94.71% | - |
| Source 19 | 12 | R1-2112573 | Genie | 0 | 0 | 0 |  | H | 5 | 5 | 94.71% | 39.55% |
| Source 15 | 41 | R1-2112572 | R15/16CDRX | 4 | 2 | 2 |  | H | 4 | 4 | 69.00% | 14.50% |
| Source 15 | 42 | R1-2112572 | R15/16CDRX | 8 | 4 | 4 |  | H | 4 | 4 | 40.00% | 10.80% |
| Source 15 | 43 | R1-2112572 | R15/16CDRX | 16 | 8 | 8 |  | H | 4 | 4 | 0.00% | 7.90% |
| Source 15 | 44 | R1-2112572 | R15/16CDRX | 10 | 8 | 2 |  | H | 4 | 4 | 88.00% | 3.20% |
| Source 15 | 45 | R1-2112572 | R15/16CDRX | 10 | 5 | 5 |  | H | 4 | 4 | 24.00% | 9.50% |
| Source 11 | 26 | R1-2111830 | AlwaysOn - baseline | - | - | - |  | H | 2 | 2 | 95.00% | 0.00% |
| Source 11 | 27 | R1-2111830 | R15/16CDRX | 16 | 12 | 4 |  | H | 2 | 2 | 63.00% | 6.30% |
| Source 11 | 28 | R1-2111830 | R15/16CDRX | 4 | 2 | 2 |  | H | 2 | 2 | 63.00% | 15.54% |
| Source 11 | 29 | R1-2111830 | R15/16CDRX | 16 | 8 | 8 |  | H | 2 | 2 | 13.00% | 9.67% |
| Source 11 | 30 | R1-2111830 | R15/16CDRX | 10 | 8 | 2 |  | H | 2 | 2 | 80.00% | 6.61% |
| Source 14 | 6 | R1-2109555 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 7 | 7 | 91.75% | 0.00% |
| Source 14 | 7 | R1-2109555 | R15/16CDRX | 10 | 5 | 5 |  | H | 7 | 7 | 68.01% | 5.73% |
| Source 14 | 8 | R1-2109555 | Custom : cross-slot + MIMO layer adaptation by BWP switching | 0 | 0 | 0 |  | H | 7 | 7 | 88.93% | 9.33% |
| Source 14 | 9 | R1-2109555 | Custom : cross-slot + MIMO layer adaptation + PDCCH skipping by BWP switching | 0 | 0 | 0 |  | H | 7 | 7 | 86.12% | 9.78% |
| Source 14 | 10 | R1-2109555 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | H | 7 | 7 | 90.00% | 12.66% |

Observations

- In FR1, DL only evaluation, DU, VR/AR45 and low load, it is observed from Source 18 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (10/8/4, 16/14/4) provides the mean power saving gain of 4.55% in the range of 3.53 ~ 5.56% with marginal loss in DL UE satisfied rate.

- In FR1, DL-only evaluation, DU, VR/AR45 and low load, it is observed from Source 18 that the R17 PDCCH monitoring adaptation scheme provides the mean power saving gain of  21.95% with marginal loss in DL UE satisfied rate.

Table 28 Source specific data: FR1, DL-only, DU, AR/VR 45Mbps, low load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 49 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | L | 3 | 6 | 100.00% | - |
| Source 18 | 50 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | L | 3 | 6 | 100.00% | 5.56% |
| Source 18 | 51 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | L | 3 | 6 | 100.00% | 3.53% |
| Source 18 | 55 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | L | 3 | 6 | 100.00% | 21.95% |
| Source 19 | 9 | R1-2112573 | AlwaysOn - baseline | 0 | 0 | 0 |  | L | 3 | 5 | 99.7% | - |
| Source 19 | 10 | R1-2112573 | Genie | 0 | 0 | 0 |  | L | 3 | 5 | 99.7% | 43.73% |

            - CG

Observations

- In FR1, DL only evaluation, DU, CG30 and high load, it is observed from Source 9, Source 15, Source 11, Source 7, Source 14 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (10/8/4, 16/14/4, 4/2/2, 8/4/4, 10/8/2, 16/12/4, 10/8/2, 10/8/3, 10/5/5) provides the mean power saving gain of 8.4% in the range of 3.3 ~ 20.0% with marginal loss in DL UE satisfied rate.

- The choice of a particular R15/16 CDRX configuration (cycle, on duration, and inactivity timer) greatly affects the PS gain.

- In FR1, DL only evaluation, DU, CG30 and high load, it is observed from Source 14 that the R17 PDCCH monitoring adaptation scheme provides the mean power saving gain of 12.86% with marginal loss in DL UE satisfied rate.

- In FR1, DL only evaluation, DU, CG30 and high load, it is observed from Source 14 that the cross-slot scheduling + MIMO layer adaptation by BWP switching provides the mean power saving gain of  8.13% with marginal loss in DL UE satisfied rate.

- In FR1, DL only evaluation, DU, CG30 and high load, it is observed from Source 14 that the cross-slot scheduling + MIMO layer adaptation + PDCCH skipping by BWP switching provides the mean power saving gain of 8.53% with marginal loss in DL UE satisfied rate.

Table 29 Source specific data: FR1, DL-only, DU, CG 30Mbps, high load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 9 | 31 | R1-2110811 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 7 | 7 | 90.88% | 0.00% |
| Source 9 | 32 | R1-2110811 | R15/16CDRX | 10 | 5 | 4 |  | H | 7 | 7 | 77.96% | 13.83% |
| Source 9 | 33 | R1-2110811 | R15/16CDRX | 10 | 8 | 4 |  | H | 7 | 7 | 90.00% | 5.26% |
| Source 9 | 34 | R1-2110811 | R15/16CDRX | 16 | 8 | 8 |  | H | 7 | 7 | 74.42% | 9.71% |
| Source 9 | 35 | R1-2110811 | R15/16CDRX | 16 | 14 | 4 |  | H | 7 | 7 | 89.96% | 3.30% |
| Source 19 | 7 | R1-2112573 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 8 | 8 | 92.88% | - |
| Source 19 | 8 | R1-2112573 | Genie | 0 | 0 | 0 |  | H | 8 | 8 | 92.88% | 39.83% |
| Source 15 | 31 | R1-2112572 | R15/16CDRX | 4 | 2 | 2 |  | H | 8 | 8 | 88.00% | 20.00% |
| Source 15 | 32 | R1-2112572 | R15/16CDRX | 8 | 4 | 4 |  | H | 8 | 8 | 84.00% | 16.70% |
| Source 15 | 33 | R1-2112572 | R15/16CDRX | 16 | 8 | 8 |  | H | 8 | 8 | 70.00% | 13.60% |
| Source 15 | 34 | R1-2112572 | R15/16CDRX | 10 | 8 | 2 |  | H | 8 | 8 | 93.00% | 8.80% |
| Source 15 | 35 | R1-2112572 | R15/16CDRX | 10 | 5 | 5 |  | H | 8 | 8 | 76.00% | 15.40% |
| Source 11 | 16 | R1-2111830 | AlwaysOn - baseline | - | - | - |  | H | 5 | 5 | 96.00% | 0.00% |
| Source 11 | 17 | R1-2111830 | R15/16CDRX | 16 | 12 | 4 |  | H | 5 | 5 | 84.50% | 6.31% |
| Source 11 | 18 | R1-2111830 | R15/16CDRX | 4 | 2 | 2 |  | H | 5 | 5 | 76.25% | 14.73% |
| Source 11 | 19 | R1-2111830 | R15/16CDRX | 16 | 8 | 8 |  | H | 5 | 5 | 64.25% | 9.53% |
| Source 11 | 20 | R1-2111830 | R15/16CDRX | 10 | 8 | 2 |  | H | 5 | 5 | 86.25% | 5.98% |
| Source 7 | 5 | R1-2112160 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 4 | 4 | 90.00% | 0.00% |
| Source 7 | 6 | R1-2112160 | Genie | 0 | 0 | 0 |  | H | 4 | 4 | 90.00% | 41.00% |
| Source 7 | 7 | R1-2112160 | R15/16CDRX | 10 | 8 | 3 |  | H | 4 | 4 | 89.00% | 4.00% |
| Source 7 | 8 | R1-2112160 | R15/16CDRX | 10 | 5 | 5 |  | H | 4 | 4 | 83.00% | 8.00% |
| Source 14 | 1 | R1-2109555 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 13 | 13 | 91.48% | 0.00% |
| Source 14 | 2 | R1-2109555 | R15/16CDRX | 10 | 5 | 5 |  | H | 13 | 13 | 80.00% | 5.63% |
| Source 14 | 3 | R1-2109555 | Custom : cross-slot + MIMO layer adaptation by BWP switching | 0 | 0 | 0 |  | H | 13 | 13 | 90.74% | 8.13% |
| Source 14 | 4 | R1-2109555 | Custom : cross-slot + MIMO layer adaptation +PDCCH skipping by BWP switching | 0 | 0 | 0 |  | H | 13 | 13 | 90.04% | 8.53% |
| Source 14 | 5 | R1-2109555 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | H | 13 | 13 | 90.29% | 12.86% |

Observations

- In FR1, DL only evaluation, DU, CG30 and low load, it is observed from Source 9, Source 10 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (10/5/4, 10/8/4, 16/8/8, 16/14/4, 8/6/6/, 8/4/6) provides the mean power saving gain of 10.3% in the range of 3.57 ~ 15.7% with marginal loss in DL UE satisfied rate.

Table 30 Source specific data: FR1, DL-only, DU, CG 30Mbps, low load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 9 | 46 | R1-2110811 | AlwaysOn - baseline | 0 | 0 | 0 |  | L | 3 | 7 | 99.68% | 0.00% |
| Source 9 | 47 | R1-2110811 | R15/16CDRX | 10 | 5 | 4 |  | L | 3 | 7 | 99.21% | 15.20% |
| Source 9 | 48 | R1-2110811 | R15/16CDRX | 10 | 8 | 4 |  | L | 3 | 7 | 99.64% | 5.75% |
| Source 9 | 49 | R1-2110811 | R15/16CDRX | 16 | 8 | 8 |  | L | 3 | 7 | 97.62% | 10.79% |
| Source 9 | 50 | R1-2110811 | R15/16CDRX | 16 | 14 | 4 |  | L | 3 | 7 | 99.64% | 3.57% |
| Source 10 | 5 | R1-2111521 | Always On | 0 | 0 | 0 |  | L | 4 | 6 | 98.00% | 0.00% |
| Source 10 | 6 | R1-2111521 | Genie | 0 | 0 | 0 |  | L | 4 | 6 | 98.00% | 58.30% |
| Source 10 | 7 | R1-2111521 | CDRX | 8 | 6 | 6 |  | L | 4 | 6 | 95.00% | 10.80% |
| Source 10 | 8 | R1-2111521 | CDRX | 8 | 4 | 6 |  | L | 4 | 6 | 92.00% | 15.70% |
| Source 19 | 5 | R1-2112573 | AlwaysOn - baseline | 0 | 0 | 0 |  | L | 4 | 8 | 100.00% | - |
| Source 19 | 6 | R1-2112573 | Genie | 0 | 0 | 0 |  | L | 4 | 8 | 100.00% | 44.24% |

###### InH

Table 31 Summary of FR1, DL-only power evaluation results for InH

| Scen-arios | App | DL Bit rate (Mbps) | PS scheme, Note 2 | System Load | PSG(%), Note 1,3 |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Mean (%) | Range (%) |  |
| InH | VR/AR | 30 | R15/16 CDRX | High | 9.67 | 2.39 ~ 20.90 | Source 18, Source 3, Source 15, Source 11, Source 12, Source 20 |
|  |  |  |  | Low | 4.7 | 3.67 ~ 5.72 | Source 18 |
|  |  |  | R17 PDCCH monitoring adaptation | High | 20.73 |  | Source 18 |
|  |  |  |  | Low | 23.33 |  | Source 18 |
|  | VR/AR | 45 | R15/16 CDRX | High | 7.61 | 2.83 ~ 15.7 | Source 18, Source 20, Source 15, Source 11 |
|  |  |  |  | low | 4.39 | 3.46 ~ 5.32 | Source 18 |
|  |  |  | R17 PDCCH monitoring adaptation | High | 17.15 | 14.41 ~ 19.89 | Source 18, Source 14 |
|  |  |  |  | Low | 22.16 |  | Source 18 |
|  |  |  | Cross-slot + MIMO layer adaptation by BWP switching | High | 8.84 |  | Source 14 |
|  |  |  | Cross-slot + MIMO layer adaptation + PDCCH skipping by BWP switching | High | 9.31 |  | Source 14 |
|  | CG | 30 | R15/16 CDRX | High | 13.11 | 4.2 ~ 20.9 | Source 15, Source 20, Source 11 |
|  |  |  |  | Low |  |  |  |
| Note 1 : PSG was computed for the cases only with marginal loss in % of DL satisfied UE. Note 2: The CDRX configurations considered in each case could be different. The details of considered R15/16 CDRX configurations in this table are listed in the following tables. Note 3: The PSG is computed with respect to power consumption of AlwaysOn scheme. |  |  |  |  |  |  |  |

            - VR/AR

Observations

- In FR1, DL only evaluation, InH, VR/AR30 and high load, it is observed from Source 18, Source 3, Source 15, Source 11, Source 12, Source 20 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (10/8/4, 16/14/4, 16/12/4, 6/4/2, 10/8/4, 4/2/2, 8/4/4, 10/8/2, 16/8/8, 16/8/4, 16/8/6, 16/10/8, 16/12/8) provides the mean power saving gain of 9.67% in the range of 2.39 ~ 20.90% with marginal loss in DL UE satisfied rate.

- The choice of a particular R15/16 CDRX configuration (cycle, on duration, and inactivity timer) greatly affects the PS gain.

- In FR1, DL only evaluation, InH, VR/AR30 and high load, it is observed from Source 18 that the R17 PDCCH monitoring adaptation scheme provides the mean power saving gain of 20.73% with marginal loss in DL UE satisfied rate.

Table 32 Source specific data: FR1, DL-only, InH, VR/AR 30Mbps, high load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 9 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 10 | 10 | 92.50% | - |
| Source 18 | 10 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | H | 10 | 10 | 91.25% | 4.88% |
| Source 18 | 11 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | H | 10 | 10 | 91.81% | 3.24% |
| Source 18 | 15 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | H | 10 | 10 | 92.17% | 20.73% |
| Source 3 | 1 | R1-2111234 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 12 | 12 | 95.83% | 0.00% |
| Source 3 | 2 | R1-2111234 | R15/16CDRX | 16 | 12 | 4 |  | H | 12 | 12 | 90.97% | 2.39% |
| Source 3 | 3 | R1-2111234 | R15/16CDRX | 6 | 4 | 2 |  | H | 12 | 12 | 88.89% | 6.14% |
| Source 20 | 27 | R1-2111351 | AlwaysOn-baseline | 0 | 0 | 0 |  | H | 11 | 11 | 93.18% | 0.00% |
| Source 20 | 28 | R1-2111351 | AlwaysOn-baseline | 0 | 0 | 0 |  | H | 10 | 11 | 93.00% | 0.00% |
| Source 20 | 29 | R1-2111351 | R15 CDRX | 10 | 8 | 4 |  | H | 11 | 11 | 90.15% | 6.20% |
| Source 15 | 6 | R1-2112572 | R15/16CDRX | 4 | 2 | 2 |  | H | 5 | 5 | 90.00% | 20.90% |
| Source 15 | 7 | R1-2112572 | R15/16CDRX | 8 | 4 | 4 |  | H | 5 | 5 | 83.00% | 18.20% |
| Source 15 | 8 | R1-2112572 | R15/16CDRX | 16 | 8 | 8 |  | H | 5 | 5 | 0.00% | 16.20% |
| Source 15 | 9 | R1-2112572 | R15/16CDRX | 10 | 8 | 2 |  | H | 5 | 5 | 93.00% | 9.30% |
| Source 15 | 10 | R1-2112572 | R15/16CDRX | 10 | 5 | 5 |  | H | 5 | 5 | 74.00% | 17.30% |
| Source 11 | 6 | R1-2111830 | AlwaysOn - baseline | - | - | - |  | H | 5 | 5 | 96.80% | 0.00% |
| Source 11 | 7 | R1-2111830 | R15/16CDRX | 16 | 12 | 4 |  | H | 5 | 5 | 58.00% | 6.14% |
| Source 11 | 8 | R1-2111830 | R15/16CDRX | 4 | 2 | 2 |  | H | 5 | 5 | 57.60% | 15.5% |
| Source 11 | 9 | R1-2111830 | R15/16CDRX | 16 | 8 | 8 |  | H | 5 | 5 | 14.40% | 8.77% |
| Source 11 | 10 | R1-2111830 | R15/16CDRX | 10 | 8 | 2 |  | H | 5 | 5 | 82.80% | 6.55% |
| Source 12 | 12 | R1-2112175 | R15/16CDRX | 4 | 2 | 2 |  | H | 4 | 4 | 97.91% | 14.86% |
| Source 12 | 13 | R1-2112175 | R15/16CDRX | 8 | 4 | 4 |  | H | 4 | 4 | 95.83% | 13.28% |
| Source 12 | 14 | R1-2112175 | R15/16CDRX | 16 | 8 | 8 |  | H | 4 | 4 | 85.42% | 10.22% |
| Source 12 | 15 | R1-2112175 | R15/16CDRX | 16 | 8 | 2 |  | H | 4 | 4 | 68.75% | 16.31% |
| Source 12 | 16 | R1-2112175 | R15/16CDRX | 16 | 8 | 4 |  | H | 4 | 4 | 81.25% | 14.79% |
| Source 12 | 17 | R1-2112175 | R15/16CDRX | 16 | 8 | 6 |  | H | 4 | 4 | 83.33% | 12.46% |
| Source 12 | 18 | R1-2112175 | R15/16CDRX | 16 | 2 | 8 |  | H | 4 | 4 | 12.50% | 27.51% |
| Source 12 | 19 | R1-2112175 | R15/16CDRX | 16 | 4 | 8 |  | H | 4 | 4 | 35.42% | 20.18% |
| Source 12 | 20 | R1-2112175 | R15/16CDRX | 16 | 6 | 8 |  | H | 4 | 4 | 58.33% | 14.87% |
| Source 12 | 21 | R1-2112175 | R15/16CDRX | 16 | 10 | 8 |  | H | 4 | 4 | 95.83% | 5.97% |
| Source 12 | 22 | R1-2112175 | R15/16CDRX | 16 | 12 | 8 |  | H | 4 | 4 | 100.00% | 5.30% |

Observations

- In FR1, DL only evaluation, InH, VR/AR30 and low load, it is observed from Source 18 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (10/8/4,16/14/4) provides the mean power saving gain of 4.7% in the range of 3.67 ~ 5.72% with marginal loss in DL UE satisfied rate.

- In FR1, DL only evaluation, InH, VR/AR30 and low load, it is observed from Source 18 that the R17 PDCCH monitoring adaptation scheme provides the mean power saving gain of 23.33% with marginal loss in DL UE satisfied rate.

Table 33 Source specific data: FR1, DL-only, InH, VR/AR 30Mbps, low load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 1 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | L | 5 | 10 | 100.00% | - |
| Source 18 | 2 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | L | 5 | 10 | 100.00% | 5.72% |
| Source 18 | 3 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | L | 5 | 10 | 100.00% | 3.67% |
| Source 18 | 7 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | L | 5 | 10 | 100.00% | 23.33% |

Observations

- In FR1, DL only evaluation, InH, VR/AR45 and high load, it is observed from Source 18, Source 20, Source 15, Source 11 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (10/8/4, 16/14/4, 10/8/4, 4/2/2, 8/4/4, 10/8/2) provides the mean power saving gain of 7.61% in the range of 2.83 ~ 15.7% with marginal loss in DL UE satisfied rate.

- In FR1, DL only evaluation, InH, VR/AR45 and high load, it is observed from Source 18, Source 14 that the R17 PDCCH monitoring adaptation scheme provides the mean power saving gain of 17.15% in the range of 14.41% ~ 19.89% with marginal loss in DL UE satisfied rate.

- In FR1, DL only evaluation, InH, VR/AR45 and high load, it is observed from Source 14 that cross-slot + MIMO layer adaptation by BWP switching scheme provides the mean power saving gain of 8.84% with marginal loss in DL UE satisfied rate.

- In FR1, DL only evaluation, InH, VR/AR45 and high load, it is observed from Source 14 that cross-slot + MIMO layer adaptation + PDCCH skipping by BWP switching scheme provides the mean power saving gain of 9.31% with marginal loss in DL UE satisfied rate.

Table 34 Source specific data: FR1, DL-only, InH, VR/AR 45Mbps, high load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 25 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 5 | 5 | 96.67% | - |
| Source 18 | 26 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | H | 5 | 5 | 92.78% | 4.68% |
| Source 18 | 27 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | H | 5 | 5 | 94.44% | 2.83% |
| Source 18 | 31 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | H | 5 | 5 | 96.67% | 19.89% |
| Source 20 | 32 | R1-2111351 | AlwaysOn-baseline | 0 | 0 | 0 |  | H | 7 | 7 | 91.00% | 0.00% |
| Source 20 | 33 | R1-2111351 | R15 CDRX | 10 | 8 | 4 |  | H | 7 | 7 | 87.00% | 7.40% |
| Source 15 | 11 | R1-2112572 | R15/16CDRX | 4 | 2 | 2 |  | H | 3 | 3 | 95.00% | 15.70% |
| Source 15 | 12 | R1-2112572 | R15/16CDRX | 8 | 4 | 4 |  | H | 3 | 3 | 84.70% | 12.10% |
| Source 15 | 13 | R1-2112572 | R15/16CDRX | 16 | 8 | 8 |  | H | 3 | 3 | 0.00% | 9.40% |
| Source 15 | 14 | R1-2112572 | R15/16CDRX | 10 | 8 | 2 |  | H | 3 | 3 | 97.00% | 4.00% |
| Source 15 | 15 | R1-2112572 | R15/16CDRX | 10 | 5 | 5 |  | H | 3 | 3 | 63.00% | 10.80% |
| Source 11 | 11 | R1-2111830 | AlwaysOn - baseline | - | - | - |  | H | 3 | 3 | 98.00% | 0.00% |
| Source 11 | 12 | R1-2111830 | R15/16CDRX | 16 | 12 | 4 |  | H | 3 | 3 | 63.34% | 5.76% |
| Source 11 | 13 | R1-2111830 | R15/16CDRX | 4 | 2 | 2 |  | H | 3 | 3 | 63.34% | 15.12% |
| Source 11 | 14 | R1-2111830 | R15/16CDRX | 16 | 8 | 8 |  | H | 3 | 3 | 14.67% | 8.53% |
| Source 11 | 15 | R1-2111830 | R15/16CDRX | 10 | 8 | 2 |  | H | 3 | 3 | 85.30% | 6.54% |
| Source 14 | 11 | R1-2109555 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 4 | 4 | 91.67% | 0.00% |
| Source 14 | 12 | R1-2109555 | R15/16CDRX | 10 | 5 | 5 |  | H | 4 | 4 | 70.83% | 4.45% |
| Source 14 | 13 | R1-2109555 | Custom : cross-slot + MIMO layer adaptation by BWP switching | 0 | 0 | 0 |  | H | 4 | 4 | 88.73% | 8.84% |
| Source 14 | 14 | R1-2109555 | Custom : cross-slot + MIMO layer adaptation +PDCCH skipping by BWP switching | 0 | 0 | 0 |  | H | 4 | 4 | 84.80% | 9.31% |
| Source 14 | 15 | R1-2109555 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | H | 4 | 4 | 90.00% | 14.41% |

Observations

- In FR1, DL only evaluation, InH, VR/AR45 and low load, it is observed from Source 18 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (10/8/4,16/14/4) provides the mean power saving gain of 4.39% in the range of 3.46 ~ 5.32% with marginal loss in DL UE satisfied rate.

- In FR1, DL only evaluation, InH, VR/AR45 and low load, it is observed from Source 18 that the R17 PDCCH monitoring adaptation scheme provides the mean power saving gain of 22.16% with marginal loss in DL UE satisfied rate.

Table 35 Source specific data: FR1, DL-only, InH, VR/AR 45Mbps, low load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 23 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | L | 3 | 5 | 100% | - |
| Source 18 | 24 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | L | 3 | 5 | 100% | 5.32% |
| Source 18 | 25 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | L | 3 | 5 | 100% | 3.46% |
| Source 18 | 29 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | L | 3 | 5 | 100% | 22.16% |

            - CG

Observations

- In FR1, DL only evaluation, InH, CG30 and high load, it is observed from Source 20, Source 15, Source 11 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (10/8/4, 4/2/2, 8/4/4, 16/8/8, 10/8/2, 10/5/5, 10/8/2) provides the mean power saving gain of 13.11% in the range of 4.2 ~ 20.90% with marginal loss in DL UE satisfied rate.

- The choice of a particular R15/16 CDRX configuration (cycle, on duration, and inactivity timer) greatly affects the PS gain.

Table 36 Source specific data: FR1, DL-only, InH, CG 30Mbps, high load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 20 | 36 | R1-2111351 | AlwaysOn-baseline | 0 | 0 | 0 |  | H | 12 | 12 | 96.53% | 0.00% |
| Source 20 | 37 | R1-2111351 | R15 CDRX | 10 | 8 | 4 |  | H | 12 | 12 | 88.88% | 4.20% |
| Source 15 | 1 | R1-2112572 | R15/16CDRX | 4 | 2 | 2 |  | H | 5 | 5 | 96.80% | 20.90% |
| Source 15 | 2 | R1-2112572 | R15/16CDRX | 8 | 4 | 4 |  | H | 5 | 5 | 96.70% | 18.20% |
| Source 15 | 3 | R1-2112572 | R15/16CDRX | 16 | 8 | 8 |  | H | 5 | 5 | 95.00% | 16.20% |
| Source 15 | 4 | R1-2112572 | R15/16CDRX | 10 | 8 | 2 |  | H | 5 | 5 | 98.50% | 9.30% |
| Source 15 | 5 | R1-2112572 | R15/16CDRX | 10 | 5 | 5 |  | H | 5 | 5 | 96.30% | 17.30% |
| Source 11 | 1 | R1-2111830 | AlwaysOn - baseline | - | - | - |  | H | 7 | 7 | 97.57% | 0.00% |
| Source 11 | 2 | R1-2111830 | R15/16CDRX | 16 | 12 | 4 |  | H | 7 | 7 | 79.90% | 5.52% |
| Source 11 | 3 | R1-2111830 | R15/16CDRX | 4 | 2 | 2 |  | H | 7 | 7 | 64.71% | 13.63% |
| Source 11 | 4 | R1-2111830 | R15/16CDRX | 16 | 8 | 8 |  | H | 7 | 7 | 55.70% | 6.95% |
| Source 11 | 5 | R1-2111830 | R15/16CDRX | 10 | 8 | 2 |  | H | 7 | 7 | 83.30% | 5.68% |
| Source 12 | 1 | R1-2112175 | R15/16CDRX | 4 | 2 | 2 |  | H | 9 | 9 | 57.40% | 14.51% |
| Source 12 | 2 | R1-2112175 | R15/16CDRX | 8 | 4 | 4 |  | H | 9 | 9 | 56.40% | 12.62% |
| Source 12 | 3 | R1-2112175 | R15/16CDRX | 16 | 8 | 8 |  | H | 9 | 9 | 46.29% | 9.74% |
| Source 12 | 4 | R1-2112175 | R15/16CDRX | 16 | 8 | 2 |  | H | 9 | 9 | 20.37% | 15.35% |
| Source 12 | 5 | R1-2112175 | R15/16CDRX | 16 | 8 | 4 |  | H | 9 | 9 | 29.63% | 13.37% |
| Source 12 | 6 | R1-2112175 | R15/16CDRX | 16 | 8 | 6 |  | H | 9 | 9 | 39.81% | 11.42% |
| Source 12 | 7 | R1-2112175 | R15/16CDRX | 16 | 2 | 8 |  | H | 9 | 9 | 5.56% | 28.88% |
| Source 12 | 8 | R1-2112175 | R15/16CDRX | 16 | 4 | 8 |  | H | 9 | 9 | 10.18% | 20.03% |
| Source 12 | 9 | R1-2112175 | R15/16CDRX | 16 | 6 | 8 |  | H | 9 | 9 | 27.78% | 14.39% |
| Source 12 | 10 | R1-2112175 | R15/16CDRX | 16 | 10 | 8 |  | H | 9 | 9 | 53.70% | 5.37% |
| Source 12 | 11 | R1-2112175 | R15/16CDRX | 16 | 12 | 8 |  | H | 9 | 9 | 70.37% | 4.92% |

No input for FR1, DL-only, CG30, low load case

###### UMa

Table 37 Summary of FR1, DL-only power evaluation results for UMa

| Scen-arios | App | DL Bit rate (Mbps) | PS scheme, Note 2 | System Load | PSG(%), Note 1,3 |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Mean (%) | Range (%) |  |
| UMa | VR/AR | 30 | R15/16 CDRX | High | 4.13 | 3.23 ~ 5.02 | Source 18 |
|  |  |  |  | Low | 5.16 | 4.05 ~ 6.26 | Source 18 |
|  |  |  | R17 PDCCH monitoring adaptation | High | 20.54 |  | Source 18 |
|  |  |  |  | Low | 25.15 |  | Source 18 |
|  |  | 45 | R15/16 CDRX | High | 4.03 | 3.13 ~ 4.92 | Source 18 |
|  |  |  |  | Low | 4.89 | 3.97 ~ 5.81 | Source 18 |
|  |  |  | R17 PDCCH monitoring adaptation | High | 20.17 |  | Source 18 |
|  |  |  |  | Low | 23.25 |  | Source 18 |
| Note 1 : PSG was computed for the cases only with marginal loss in % of DL satisfied UE. Note 2: The CDRX configurations considered in each case could be different. The details of considered R15/16 CDRX configurations in this table are listed in the following tables. Note 3: The PSG is computed with respect to power consumption of AlwaysOn scheme. |  |  |  |  |  |  |  |

            - VR/AR

Observations

- In FR1, DL only evaluation, UMa, VR/AR30 and high load, it is observed from Source 18 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (10/8/8, 16/14/4) provides the mean power saving gain of 4.13% in the range of 3.23 ~ 5.02% with marginal loss in DL UE satisfied rate.

- In FR1, DL only evaluation, UMa, VR/AR30 and high load, it is observed from Source 18 that the R17 PDCCH monitoring adaptation scheme provides the mean power saving gain of 20.54% with marginal loss in DL UE satisfied rate.

Table 38 Source specific data: FR1, DL-only, UMa, VR/AR, 30Mbps, high load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 73 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 8 | 8 | 93.75% | - |
| Source 18 | 74 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | H | 8 | 8 | 91.47% | 5.02% |
| Source 18 | 75 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | H | 8 | 8 | 92.85% | 3.23% |
| Source 18 | 79 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | H | 8 | 8 | 93.75% | 20.54% |

Observations

- In FR1, DL only evaluation, UMa, VR/AR30 and low load, it is observed from Source 18 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (10/8/4, 16/14/4) provides the mean power saving gain of 5.16% in the range of 4.05 ~ 6.26% with marginal loss in DL UE satisfied rate.

- In FR1, DL only evaluation, UMa, VR/AR30 and low load, it is observed from Source 18 that the R17 PDCCH monitoring adaptation scheme provides the mean power saving gain of 25.15% with marginal loss in DL UE satisfied rate.

Table 39 Source specific data: FR1, DL-only, UMa, VR/AR, 30Mbps, low load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 65 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | L | 4 | 8 | 98.81% | - |
| Source 18 | 66 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | L | 4 | 8 | 98.41% | 6.26% |
| Source 18 | 67 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | L | 4 | 8 | 98.81% | 4.05% |
| Source 18 | 71 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | L | 4 | 8 | 98.81% | 25.15% |

Observations

- In FR1, DL only evaluation, UMa, VR/AR45 and high load, it is observed from Source 18 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (10/8/4, 16/14/4) provides the mean power saving gain of 4.03% in the range of 3.13 ~ 4.92% with marginal loss in DL UE satisfied rate.

- In FR1, DL only evaluation, UMa, VR/AR45 and high load, it is observed from Source 18 that the R17 PDCCH monitoring adaptation scheme provides the mean power saving gain of 20.17% with marginal loss in DL UE satisfied rate.

Table 40 Source specific data: FR1, DL-only, UMa, VR/AR, 45Mbps, high load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 89 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 4 | 4 | 94.05% | - |
| Source 18 | 90 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | H | 4 | 4 | 92.46% | 4.92% |
| Source 18 | 91 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | H | 4 | 4 | 93.25% | 3.13% |
| Source 18 | 95 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | H | 4 | 4 | 93.33% | 20.17% |

Observations

- In FR1, DL only evaluation, UMa, VR/AR45 and low load, it is observed from Source 18 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (10/8/4/, 16/14/4) provides the mean power saving gain of 4.89% in the range of 3.97 ~ 5.81% with marginal loss in DL UE satisfied rate.

- In FR1, DL only evaluation, UMa, VR/AR45 and low load, it is observed from Source 18 that the R17 PDCCH monitoring adaptation scheme provides the mean power saving gain of 23.25% with marginal loss in DL UE satisfied rate.

Table 41 Source specific data: FR1, DL-only, UMa, VR/AR, 45Mbps, low load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 81 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | L | 2 | 4 | 96.83% | - |
| Source 18 | 82 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | L | 2 | 4 | 96.83% | 5.81% |
| Source 18 | 83 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | L | 2 | 4 | 96.83% | 3.97% |
| Source 18 | 87 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | L | 2 | 4 | 96.83% | 23.25% |

            - CG

No results were submitted

##### UL-only Evaluation

###### DU

Table 42 Summary of FR1, UL-only power evaluation results for DU

| Scen-arios | App | UL Bit rate (Mbps) | PS scheme, Note 2 | System Load | PSG(%), Note 1,3 |  | source |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Mean (%) | Range (%) |  |
| DU | VR/CG UL Pose | 0.2 | R15/16 CDRX | High | 31.95 | 26.62 ~ 37.27 | Source 18 |
|  |  |  |  | Low |  |  |  |
|  |  | 0.048 | R16 cross slot scheduling | High | 20.84 |  | Source 14 |
|  |  |  | R17 PDCCH monitoring adaptation | High | 15.32 |  | Source 14 |
|  |  |  | R17 PDCCH monitoring adaptation + R16 cross slot scheduling | High | 28.58 |  | Source 14 |
|  | AR UL 1 stream (scene) | 10 | R15/16 CDRX | High | 8.48 | 4.25 ~ 14.6 | Source 18, Source 15 |
|  |  |  |  | Low | 5.62 | 4.26 ~ 6.97 | Source 18 |
|  |  |  | R17 PDCCH monitoring adaptation | High | 26.76 | 19.36 ~ 34.15 | Source 18, Source 14 |
|  |  |  |  | Low | 35.84 |  | Source 18 |
|  |  |  | R16 cross slot scheduling | high | 24.33 |  | Source 14 |
|  |  |  | R17 PDCCH monitoring adaptation + cross slot scheduling | high | 32.80 |  | Source 14 |
|  | AR UL 2 streams (pose, scene) | 10.2 | R15/16 CDRX | High | 2.17 | 1.99 ~ 3.43 | Source 18 |
|  |  |  |  | Low | 2.51 | 1.79 ~ 3.23 | Source 18 |
|  |  |  | R17 PDCCH monitoring adaptation | High | 23.02 |  | Source 18 |
|  |  |  |  | Low | 24.16 |  | Source 18 |
| Note 1 : PSG was computed for the cases only with marginal loss in % of UL satisfied UE. Note 2: The CDRX configurations considered in each case could be different. The details of considered R15/16 CDRX configurations in this table are listed in the following tables. Note 3: The PSG is computed with respect to power consumption of AlwaysOn scheme. |  |  |  |  |  |  |  |

            - VR/CG

Observations

- In FR1, UL only evaluation, DU, VR/CG UL pose (250Hz) and high load, it is observed from Source 18 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (4/2/1, 8/3/1) provides the mean power saving gain of 31.95% in the range of 26.62 ~ 37.27% with marginal loss in UL UE satisfied rate.

- In FR1, UL only evaluation, DU, VR/CG UL pose (60Hz) and high load, it is observed from Source 14 that the R16 cross slot scheduling scheme provides the mean power saving gain of 20.48% with marginal loss in UL UE satisfied rate.

- In FR1, UL only evaluation, DU, VR/CG UL pose (60Hz) and high load, it is observed from Source 14 that the R17 PDCCH monitoring adaptation scheme provides the mean power saving gain of 15.32% with marginal loss in UL UE satisfied rate.

- In FR1, UL only evaluation, DU, VR/CG UL pose (60Hz) and high load, it is observed from Source 14 that the R17 PDCCH monitoring adaptation + cross slot scheduling scheme provide the mean power saving gain of 28.58% with marginal loss in UL UE satisfied rate.

Table 43 Source specific data: FR1, UL-only, DU, VR/CG-Pose only, 0.2Mbps, high load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 158 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 | UL pose rate 250Hz | H | 20 | 20 | 99.99% | - |
| Source 18 | 159 | R1-2111046 | R15/16CDRX | 4 | 2 | 1 | UL pose rate 250Hz | H | 20 | 20 | 94.84% | 26.62% |
| Source 18 | 160 | R1-2111046 | R15/16CDRX | 8 | 3 | 1 | UL pose rate 250Hz | H | 20 | 20 | 93.81% | 37.27% |
| Source 14 | 16 | R1-2109555 | AlwaysOn - baseline | 0 | 0 | 0 | UL pose rate 60Hz | H | 13 | 13 | 100.00% | 0.00% |
| Source 14 | 17 | R1-2109555 | Cross slot scheduling | 0 | 0 | 0 | UL pose rate 60Hz | H | 13 | 13 | 100.00% | 20.48% |
| Source 14 | 18 | R1-2109555 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 | UL pose rate 60Hz | H | 13 | 13 | 100.00% | 15.32% |
| Source 14 | 19 | R1-2109555 | Custom : R17 PDCCH monitoring adaptation + cross slot | 0 | 0 | 0 | UL pose rate 60Hz | H | 13 | 13 | 100.00% | 28.58% |

            - AR

              - AR with UL 1 stream

Observations

- In FR1, UL only evaluation, DU, AR UL 1 stream and high load, it is observed from Source 18, Source 15 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (10/8/4, 16/14/4, 4/2/2, 8/4/4, 16/8/8, 10/8/2, 10/5/5) provides the mean power saving gain of 8.48% in the range of 4.25 ~ 14.60% with marginal loss in UL UE satisfied rate.

- The choice of a particular R15/16 CDRX configuration (cycle, on duration, and inactivity timer) greatly affects the PS gain.

- In FR1, UL only evaluation, DU, AR UL 1 stream and high load, it is observed from Source 18, Source 14 that the R17 PDCCH monitoring adaptation scheme provides the mean power saving gain of 26.76% in the range of 19.36 ~ 34.15% with marginal loss in UL UE satisfied rate.

- In FR1, UL only evaluation, DU, AR UL 1 stream and high load, it is observed from Source  Source 14 that the cross-slot scheduling scheme provides the mean power saving gain of 24.33% with marginal loss in UL UE satisfied rate.

- In FR1, UL only evaluation, DU, AR UL 1 stream and high load, it is observed from Source  Source 14 that the R17 PDCCH skipping + cross slot scheduling scheme provides the mean power saving gain of 32.80% with marginal loss in UL UE satisfied rate.

Table 44 Source specific data: FR1, UL-only, DU, AR UL 1 stream, high load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 166 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 9 | 9 | 92.95% | - |
| Source 18 | 167 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | H | 9 | 9 | 91.53% | 6.73% |
| Source 18 | 168 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | H | 9 | 9 | 91.17% | 4.25% |
| Source 18 | 170 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | H | 9 | 9 | 91.77% | 34.15% |
| Source 15 | 46 | R1-2112572 | R15/16CDRX | 4 | 2 | 2 |  | H | 4 | 4 | 91.20% | 14.60% |
| Source 15 | 47 | R1-2112572 | R15/16CDRX | 8 | 4 | 4 |  | H | 4 | 4 | 91.40% | 10.80% |
| Source 15 | 48 | R1-2112572 | R15/16CDRX | 16 | 8 | 8 |  | H | 4 | 4 | 91.40% | 7.50% |
| Source 15 | 49 | R1-2112572 | R15/16CDRX | 10 | 8 | 2 |  | H | 4 | 4 | 91.30% | 5.80% |
| Source 15 | 50 | R1-2112572 | R15/16CDRX | 10 | 5 | 5 |  | H | 4 | 4 | 91.40% | 9.70% |
| Source 14 | 24 | R1-2109555 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 6 | 6 | 100.00% | 0.00% |
| Source 14 | 25 | R1-2109555 | Cross slot scheduling | 0 | 0 | 0 |  | H | 6 | 6 | 100.00% | 24.33% |
| Source 14 | 26 | R1-2109555 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | H | 6 | 6 | 100.00% | 19.36% |
| Source 14 | 27 | R1-2109555 | Custom : R17 PDCCH monitoring adaptation + cross slot | 0 | 0 | 0 |  | H | 6 | 6 | 100.00% | 32.80% |

Observations

- In FR1, UL only evaluation, DU, AR UL 1 stream and low load, it is observed from Source 18 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (10/8/4, 16/14/4) provides the mean power saving gain of 5.62% in the range of 4.26 ~ 6.97% with marginal loss in UL UE satisfied rate.

- In FR1, UL only evaluation, DU, AR UL 1 stream and low load, it is observed from Source 18 that the R16 PDCCH skipping scheme provides the mean power saving gain of 35.84% with marginal loss in UL UE satisfied rate.

Table 45 Source specific data: FR1, UL-only, DU, AR UL  1 stream, low load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 161 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | L | 5 | 9 | 97.14% | - |
| Source 18 | 162 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | L | 5 | 9 | 97.14% | 6.97% |
| Source 18 | 163 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | L | 5 | 9 | 97.14% | 4.26% |
| Source 18 | 165 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | L | 5 | 9 | 96.51% | 35.84% |

              - AR with UL 2 streams

Observations

- In FR1, UL only evaluation, DU, AR UL 2 stream and high load, it is observed from Source 18 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (10/8/4, 16/14/4) provides the mean power saving gain of 2.17% in the range of 1.99 ~ 3.43% with marginal loss in UL UE satisfied rate.

- In FR1, UL only evaluation, DU, AR UL 2 stream and high load, it is observed from Source 18 that the R17 PDCCH monitoring adaptation scheme provides the mean power saving gain of 23.02% with marginal loss in UL UE satisfied rate.

Table 46 Source specific data: FR1, UL-only, DU, AR 2 streams, high load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 215 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 7 | 7 | 92.29% | - |
| Source 18 | 216 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | H | 7 | 7 | 90.70% | 3.43% |
| Source 18 | 217 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | H | 7 | 7 | 92.06% | 1.99% |
| Source 18 | 219 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | H | 7 | 7 | 91.16% | 23.02% |

Observations

- In FR1, UL only evaluation, DU, AR UL 2 stream and low load, it is observed from Source 18 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (10/8/4, 16/14/4) provides the mean power saving gain of 2.51% in the range of 1.79 ~ 3.23% with marginal loss in UL UE satisfied rate.

- In FR1, UL only evaluation, DU, AR UL 2 stream and low load, it is observed from Source 18 that the R17 PDCCH monitoring adaptation scheme provides the mean power saving gain of 24.16% with marginal loss in UL UE satisfied rate.

Table 47 Source specific data: FR1, UL-only, DU, AR 2 streams, low load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 210 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | L | 4 | 7 | 100.00% | - |
| Source 18 | 211 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | L | 4 | 7 | 100.00% | 3.23% |
| Source 18 | 212 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | L | 4 | 7 | 100.00% | 1.79% |
| Source 18 | 214 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | L | 4 | 7 | 100.00% | 24.16% |

###### InH

Table 48 Summary of FR1, UL-only power evaluation results for InH

| Scen-arios | App | UL Bit rate (Mbps) | PS scheme, Note 2 | System Load | PSG(%), Note 1,3 |  | source |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Mean (%) | Range (%) |  |
| InH | VR/CG UL Pose | 0.2 | R15/16 CDRX | High | 31.58 | 26.33 ~ 36.83 | Source 18 |
|  |  |  |  | Low |  |  |  |
|  |  | 0.048 | Cross slot scheduling | High | 20.56 |  | Source 14 |
|  |  |  |  | Low |  |  |  |
|  |  |  | R17 PDCCH monitoring adaptation | High | 15.29 |  | Source 14 |
|  |  |  |  | Low |  |  |  |
|  |  |  | R17 PDCCH monitoring adaptation + cross slot scheduling | High | 28.60 |  | Source 14 |
|  |  |  |  | Low |  |  |  |
|  | AR UL 1 stream (scene) | 10 | R15/16 CDRX | High | 13.04 | 4.8 ~ 21.64 | Source 18, Source 15 |
|  |  |  |  | Low | 6.60 | 5.03 ~ 8.17 | Source 18 |
|  |  |  | R17 PDCCH monitoring adaptation | High | 28.43 | 17.63 ~ 39.21 | Source 18, Source 14 |
|  |  |  |  | Low | 41.99 |  | Source 18 |
|  |  |  | R16 cross slot scheduling | High | 23.87 |  | Source 14 |
|  |  |  |  | Low |  |  |  |
|  |  |  | R17 PDCCH monitoring adaptation + R16 cross slot scheduling | High | 31.56 |  | Source 14 |
|  |  |  |  | Low |  |  |  |
|  | AR UL 2 streams (pose, scene) | 10.2 | R15/16 CDRX | High | 3.16 | 2.34 ~ 3.97 | Source 18 |
|  |  |  |  | Low | 3.6 | 2.38 ~ 4.82 | Source 18 |
|  |  |  | R17 PDCCH monitoring adaptation | High | 25.63 |  | Source 18 |
|  |  |  |  | Low | 28.15 |  | Source 18 |
| Note 1 : PSG was computed for the cases only with marginal loss in % of UL satisfied UE. Note 2: The CDRX configurations considered in each case could be different. The details of considered R15/16 CDRX configurations in this table are listed in the following tables. Note 3: The PSG is computed with respect to power consumption of AlwaysOn scheme. |  |  |  |  |  |  |  |

            - VR/CG

Observations

- In FR1, UL only evaluation, InH, VR/CG UL pose (250Hz) and high load, it is observed from Source 18 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (4/2/1, 8/3/1) provides the mean power saving gain of 31.58% in the range of 26.33 ~ 36.83% with marginal loss in UL UE satisfied rate.

- In FR1, UL only evaluation, InH, VR/CG UL pose (60Hz) and high load, it is observed from Source 14 that the cross-slot scheduling scheme provides the mean power saving gain of 20.56% with marginal loss in UL UE satisfied rate.

- In FR1, UL only evaluation, InH, VR/CG UL pose (60Hz) and high load, it is observed from Source 14 that the R17 PDCCH monitoring adaptation scheme provides the mean power saving gain of 15.29% with marginal loss in UL UE satisfied rate.

- In FR1, UL only evaluation, InH, VR/CG UL pose (60Hz) and high load, it is observed from Source 14 that the R17 PDCCH monitoring adaptation + cross slot scheduling scheme provides the mean power saving gain of 28.60% with marginal loss in UL UE satisfied rate.

Table 49 Source specific data: FR1, UL-only, InH, VR/CG Pose only, high load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 145 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 | UL Pose rate 250Hz | H | 20 | 20 | 100.00% | - |
| Source 18 | 146 | R1-2111046 | R15/16CDRX | 4 | 2 | 1 | UL Pose rate 250Hz | H | 20 | 20 | 94.31% | 26.33% |
| Source 18 | 147 | R1-2111046 | R15/16CDRX | 8 | 3 | 1 | UL Pose rate 250Hz | H | 20 | 20 | 93.33% | 36.83% |
| Source 14 | 20 | R1-2109555 | AlwaysOn - baseline | 0 | 0 | 0 | UL Pose rate 60Hz | H | 9 | 9 | 100.00% | 0.00% |
| Source 14 | 21 | R1-2109555 | Cross slot scheduling | 0 | 0 | 0 | UL Pose rate 60Hz | H | 9 | 9 | 100.00% | 20.56% |
| Source 14 | 22 | R1-2109555 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 | UL Pose rate 60Hz | H | 9 | 9 | 100.00% | 15.29% |
| Source 14 | 23 | R1-2109555 | Custom : R17 PDCCH skipping + cross slot | 0 | 0 | 0 | UL Pose rate 60Hz | H | 9 | 9 | 100.00% | 28.60% |

            - AR

              - AR with UL 1 stream

Observations

- In FR1, UL only evaluation, InH, AR UL 1 stream and high load, it is observed from Source 18, Source 15 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (10/8/4, 16/14/4, 4/2/2, 8/4/4, 16/8/8/, 10/8/2, 10/5/5) provides the mean power saving gain of 13.04% in the range of 4.8 ~ 21.64% with marginal loss in UL UE satisfied rate.

- The choice of a particular R15/16 CDRX configuration (cycle, on duration, and inactivity timer) greatly affects the PS gain.

- In FR1, UL only evaluation, InH, AR UL 1 stream and high load, it is observed from Source 18, Source 14 that the R17 PDCCH monitoring adaptation scheme provides the mean power saving gain of 28.43% in the range of 17.65 ~ 39.21% with marginal loss in UL UE satisfied rate.

- In FR1, UL only evaluation, InH, AR UL 1 stream and high load, it is observed from Source 14 that the R16 cross slot scheduling scheme provides the mean power saving gain of 23.87% with marginal loss in UL UE satisfied rate.

- In FR1, UL only evaluation, InH, AR UL 1 stream and high load, it is observed from Source 14 that the R17 PDCCH monitoring adaptation + R16 cross slot scheduling scheme provides the mean power saving gain of 31.56% with marginal loss in UL UE satisfied rate.

Table 50 Source specific data: FR1, UL-only, InH, AR UL 1 stream, high load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 153 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 13 | 13 | 93.59% | - |
| Source 18 | 154 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | H | 13 | 13 | 92.22% | 7.71% |
| Source 18 | 155 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | H | 13 | 13 | 92.86% | 4.80% |
| Source 18 | 157 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | H | 13 | 13 | 92.65% | 39.21% |
| Source 15 | 51 | R1-2112572 | R15/16CDRX | 4 | 2 | 2 |  | H | 4 | 4 | 99.00% | 21.64% |
| Source 15 | 52 | R1-2112572 | R15/16CDRX | 8 | 4 | 4 |  | H | 4 | 4 | 99.00% | 18.27% |
| Source 15 | 53 | R1-2112572 | R15/16CDRX | 16 | 8 | 8 |  | H | 4 | 4 | 99.00% | 13.50% |
| Source 15 | 54 | R1-2112572 | R15/16CDRX | 10 | 8 | 2 |  | H | 4 | 4 | 99.00% | 8.67% |
| Source 15 | 55 | R1-2112572 | R15/16CDRX | 10 | 5 | 5 |  | H | 4 | 4 | 99.00% | 16.67% |
| Source 14 | 28 | R1-2109555 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 4 | 4 | 100.00% | 0.00% |
| Source 14 | 29 | R1-2109555 | Cross slot scheduling | 0 | 0 | 0 |  | H | 4 | 4 | 100.00% | 23.87% |
| Source 14 | 30 | R1-2109555 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | H | 4 | 4 | 100.00% | 17.65% |
| Source 14 | 31 | R1-2109555 | Custom : R17 PDCCH skipping + cross slot | 0 | 0 | 0 |  | H | 4 | 4 | 100.00% | 31.56% |

Observations

- In FR1, UL only evaluation, InH, AR UL 1 stream and low load, it is observed from Source 18 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (10/8/4, 16/14/4) provides the mean power saving gain of 6.60% in the range of 5.03 ~ 8.17% with marginal loss in UL UE satisfied rate.

- In FR1, UL only evaluation, InH, AR UL 1 stream and low load, it is observed from Source 18 that the R17 PDCCH monitoring adaptation scheme provides the mean power saving gain of 41.99% with marginal loss in UL UE satisfied rate.

Table 51 Source specific data: FR1, UL-only, InH, AR UL 1 stream, low load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 148 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | L | 7 | 13 | 100% | - |
| Source 18 | 149 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | L | 7 | 13 | 100% | 8.17% |
| Source 18 | 150 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | L | 7 | 13 | 100% | 5.03% |
| Source 18 | 152 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | L | 7 | 13 | 100% | 41.99% |

              - AR with UL 2 streams

Observations

- In FR1, UL only evaluation, InH, AR UL 2 streams and high load, it is observed from Source 18 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (10/8/4, 16/14/4) provides the mean power saving gain of 3.16% in the range of 2.34 ~ 3.97% with marginal loss in UL UE satisfied rate.

- In FR1, UL only evaluation, InH, AR UL 2 streams and high load, it is observed from Source 18 that the R17 PDCCH monitoring adaptation scheme provides the mean power saving gain of 25.63% with marginal loss in UL UE satisfied rate.

Table 52 Source specific data: FR1, UL-only, InH, AR UL 2 stream, high load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 205 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 12 | 12 | 93.29% | - |
| Source 18 | 206 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | H | 12 | 12 | 92.13% | 3.97% |
| Source 18 | 207 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | H | 12 | 12 | 92.59% | 2.34% |
| Source 18 | 209 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | H | 12 | 12 | 92.36% | 25.63% |

Observations

- In FR1, UL only evaluation, InH, AR UL 2 streams and low load, it is observed from Source 18 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (10/8/4, 16/14/4) provides the mean power saving gain of 3.6% in the range of 2.38 ~ 4.82% with marginal loss in UL UE satisfied rate.

- In FR1, UL only evaluation, InH, AR UL 2 streams and low load, it is observed from Source 18 that the R17 PDCCH monitoring adaptation scheme provides the mean power saving gain of 28.15% with marginal loss in UL UE satisfied rate.

Table 53 Source specific data: FR1, UL-only, InH, AR UL 2 stream, low load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 200 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | L | 6 | 12 | 100.00% | - |
| Source 18 | 201 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | L | 6 | 12 | 100.00% | 4.82% |
| Source 18 | 202 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | L | 6 | 12 | 100.00% | 2.38% |
| Source 18 | 204 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | L | 6 | 12 | 100.00% | 28.15% |

###### UMa

Table 54 Summary of FR1, UL-only power evaluation results for UMa

| Scen-arios | App | UL Bit rate (Mbps) | PS scheme | System Load | PSG(%), Note 1,2 |  | source |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Mean (%) | Range (%) |  |
| UMa | VR/CG UL Pose | 0.2 | R15/16 CDRX | High | 33.52 | 28.1 ~ 38.93 | Source 18 |
|  |  |  |  | Low |  |  |  |
| Note 1 : PSG was computed for the cases only with marginal loss in % of UL satisfied UE. Note 2: The PSG is computed with respect to power consumption of AlwaysOn scheme. |  |  |  |  |  |  |  |

            - VR/CG

Observations

- In FR1, UL only evaluation, UMa, VR/CG Pose only and high load, it is observed from Source 18 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (4/2/1, 8/3/1) provides the mean power saving gain of 33.52% in the range of 28.10 ~ 38.93% with marginal loss in UL UE satisfied rate.

Table 55 Source specific data: FR1, UL-only, UMa, VR/CG Pose only(250Hz), 0.2Mbps, high load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 171 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 20 | 20 | 97.70% | - |
| Source 18 | 172 | R1-2111046 | R15/16CDRX | 4 | 2 | 1 |  | H | 20 | 20 | 94.37% | 28.10% |
| Source 18 | 173 | R1-2111046 | R15/16CDRX | 8 | 3 | 1 |  | H | 20 | 20 | 92.94% | 38.93% |

            - AR

No results were submitted.

#### FR2

##### DL+UL Evaluation

No results submitted.

##### DL-only Evaluation

###### DU

Table 56 Summary of FR2, DL-only power evaluation results for DU

| Scen-arios | App | DL Bit rate (Mbps) | PS scheme, Note 2 | System Load | PSG(%), Note 1,3 |  | source |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Mean (%) | Range (%) |  |
| DU | VR/AR | 30 | R15/16 CDRX | High | 7.73 | 5.96 ~ 9.5 | Source 18, Source 16 |
|  |  |  |  | Low | 8.28 | 6.4 ~ 10.15 | Source 18 |
|  |  |  | R17 PDCCH monitoring adaptation | High | 31.24 |  | Source 18 |
|  |  |  |  | Low | 31.74 |  | Source 18 |
|  |  | 45 | R15/16 CDRX | High | 6.64 | 4.98 ~ 8.29 | Source 18 |
|  |  |  |  | Low | 7.63 | 6.06 ~ 9.2 | Source 18 |
|  |  |  | R17 PDCCH monitoring adaptation | High | 26.33 |  | Source 18 |
|  |  |  |  | Low | 28.25 |  | Source 18 |
| Note 1 : PSG was computed for the cases only with marginal loss in % of DL satisfied UE. Note 2: The CDRX configurations considered in each case could be different. The details of considered R15/16 CDRX configurations in this table are listed in the following tables. Note 3: The PSG is computed with respect to power consumption of AlwaysOn scheme. |  |  |  |  |  |  |  |

            - VR/AR

Observations

- In FR2, DL only evaluation, DU, VR/AR30 and high load, it is observed from Source 18 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (10/8/4, 16/14/4) provides the mean power saving gain of 7.73% in the range of 5.96 ~ 9.5% with marginal loss in DL UE satisfied rate.

- In FR2, DL only evaluation, DU, VR/AR30 and high load, it is observed from Source 18 that the R17 PDCCH monitoring adaptation scheme provides the mean power saving gain of 31.24% with marginal loss in DL UE satisfied rate.

Table 57 Source specific data: FR2, DL-only, DU, VR/AR30, high load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 127 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 13 | 13 | 95.24% | - |
| Source 18 | 128 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | H | 13 | 13 | 91.82% | 9.50% |
| Source 18 | 129 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | H | 13 | 13 | 93.53% | 5.96% |
| Source 18 | 131 | R1-2111046 | R17 PDCCH skipping | 0 | 0 | 0 |  | H | 13 | 13 | 95.00% | 31.24% |
| Source 16 | 71 | R1-2112720 | ALWAYS ON | None | None | None |  | H | 7 | 7 | 90.00% | 0.00% |
| Source 16 | 72 | R1-2112720 | CDRX | 16 | 4 | 4 |  | H | 7 | 7 | 0.00% | 26.77% |
| Source 16 | 73 | R1-2112720 | CDRX | 16 | 8 | 8 |  | H | 7 | 7 | 45.00% | 8.77% |
| Source 16 | 74 | R1-2112720 | CDRX | 16 | 8 | 16 |  | H | 7 | 7 | 60.00% | 3.34% |
| Source 16 | 75 | R1-2112720 | Genie (CDRX with ideal PDCCH Skipping) | 16 | None | none | Genie is the same for all CDRX | H | 7 | 7 | 90.00% | 69.00% |

Observations

- In FR2, DL only evaluation, DU, VR/AR30 and low load, it is observed from Source 18 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (10/8/4, 16/14/4) provides the mean power saving gain of 8.28% in the range of 6.4 ~ 10.15% with marginal loss in DL UE satisfied rate.

- In FR2, DL only evaluation, DU, VR/AR30 and low load, it is observed from Source 18 that the R17 PDCCH monitoring adaptation scheme provides the mean power saving gain of 31.74% with marginal loss in DL UE satisfied rate.

Table 58 Source specific data: FR2, DL-only, DU, VR/AR30, low load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 121 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | L | 7 | 13 | 99.55% | - |
| Source 18 | 122 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | L | 7 | 13 | 98.64% | 10.15% |
| Source 18 | 123 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | L | 7 | 13 | 99.32% | 6.40% |
| Source 18 | 125 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | L | 7 | 13 | 99.32% | 31.74% |

Observations

- In FR2, DL only evaluation, DU, VR/AR45 and high load, it is observed from Source 18 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (10/8/4) provides the mean power saving gain of 6.64% in the range of 4.98 ~ 8.29% with marginal loss in DL UE satisfied rate.

- In FR2, DL only evaluation, DU, VR/AR45 and high load, it is observed from Source 18 that the R17 PDCCH monitoring adaptation scheme provides the mean power saving gain of 26.33% with marginal loss in DL UE satisfied rate.

Table 59 Source specific data: FR2, DL-only, DU, VR45, high load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 139 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 8 | 8 | 93.25% | - |
| Source 18 | 140 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | H | 8 | 8 | 91.67% | 8.29% |
| Source 18 | 141 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | H | 8 | 8 | 32.26% | 4.98% |
| Source 18 | 143 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | H | 8 | 8 | 93.25% | 26.33% |

Observations

- In FR2, DL only evaluation, DU, VR/AR45 and low load, it is observed from Source 18 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (10/8/4, 16/14/4) provides the mean power saving gain of 7.63% in the range of 6.06 ~ 9.2% with marginal loss in DL UE satisfied rate.

- In FR2, DL only evaluation, DU, VR/AR45 and low load, it is observed from Source 18 that the R17 PDCCH monitoring adaptation scheme provides the mean power saving gain of 28.25% with marginal loss in DL UE satisfied rate.

Table 60 Source specific data: FR2, DL-only, DU, VR45, low load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 133 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | L | 4 | 8 | 100.00% | - |
| Source 18 | 134 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | L | 4 | 8 | 100.00% | 9.20% |
| Source 18 | 135 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | L | 4 | 8 | 100.00% | 6.06% |
| Source 18 | 137 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | L | 4 | 8 | 100.00% | 28.25% |

            - CG

No results available

###### InH

Table 61 Summary of FR2, DL-only power evaluation results for InH

| Scen-arios | App | DL Bit rate (Mbps) | PS scheme, Note 2 | System Load | PSG (%), Note 1,3 |  | source |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Mean (%) | Range (%) |  |
| InH | VR/AR | 30 | R15/16 CDRX | High | 10.78 | 5.81 ~ 19.58 | Source 18, Source 15, Source 16 |
|  |  |  |  | Low | 8.17 | 6.28 ~ 10.06 | Source 18 |
|  |  |  | R17 PDCCH monitoring adaptation | High | 32.69 |  | Source 18 |
|  |  |  |  | Low | 33.80 |  | Source 18 |
|  |  | 45 | R15/16 CDRX | High | 7.46 | 5.73 ~ 18.00 | Source 18, Source 15 |
|  |  |  |  | Low | 7.75 | 5.98 ~ 9.52 | Source 18 |
|  |  |  | R17 PDCCH monitoring adaptation | High | 28.58 | 27.36 ~ 29.8 | Source 18, Source 16 |
|  |  |  |  | Low | 28.87 |  | Source 18 |
|  |  |  | R16 cross slot scheduling | High | 12.20 |  | Source 16 |
|  |  |  | R17 PDCCH monitoring adaptation + cross slot scheduling | High | 30 |  | Source 16 |
|  | CG | 30 | R15/16 CDRX | High | 13.03 | 3.79 ~ 22.66 | Source 15, Source 16 |
|  |  |  |  | Low |  |  |  |
| Note 1 : PSG was computed for the cases only with marginal loss in % of DL satisfied UE. Note 2: The CDRX configurations considered in each case could be different. The details of considered R15/16 CDRX configurations in this table are listed in the following tables. Note 3: The PSG is computed with respect to power consumption of AlwaysOn scheme. |  |  |  |  |  |  |  |

            - VR/AR

Observations

- In FR2, DL only evaluation, InH, VR/AR30 and high load, it is observed from Source 18, Source 15, Source 16 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (10/8/4, 16/14/4, 4/2/2, 10/8/2) provides the mean power saving gain of 10.78% in the range of 5.81 ~ 19.58% with marginal loss in DL UE satisfied rate.

- The choice of a particular R15/16 CDRX configuration (cycle, on duration, and inactivity timer) greatly affects the PS gain.

- In FR2, DL only evaluation, DU, VR/AR30 and high load, it is observed from Source 18 that the R17 PDCCH monitoring adaptation scheme provides the mean power saving gain of 32.69% with marginal loss in DL UE satisfied rate.

Table 62 Source specific data: FR2, DL-only, InH, VR/AR30, high load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 103 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 8 | 8 | 92.01% | - |
| Source 18 | 104 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | H | 8 | 8 | 90.63% | 9.53% |
| Source 18 | 105 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | H | 8 | 8 | 91.37% | 5.81% |
| Source 18 | 107 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | H | 8 | 8 | 92.01% | 32.69% |
| Source 15 | 21 | R1-2112572 | R15/16CDRX | 4 | 2 | 2 |  | H | 10 | 10 | 85.58% | 19.58% |
| Source 15 | 22 | R1-2112572 | R15/16CDRX | 8 | 4 | 4 |  | H | 10 | 10 | 20.66% | 16.41% |
| Source 15 | 23 | R1-2112572 | R15/16CDRX | 16 | 8 | 8 |  | H | 10 | 10 | 0.00% | 13.16% |
| Source 15 | 24 | R1-2112572 | R15/16CDRX | 10 | 8 | 2 |  | H | 10 | 10 | 92.41% | 8.21% |
| Source 15 | 25 | R1-2112572 | R15/16CDRX | 10 | 5 | 5 |  | H | 10 | 10 | 7.16% | 14.92% |
| Source 16 | 66 | R1-2112720 | ALWAYS ON | None | None | None |  | H | 7 | 7 | 90.00% | 0.00% |
| Source 16 | 67 | R1-2112720 | CDRX | 16 | 4 | 4 |  | H | 7 | 7 | 0.00% | 28.44% |
| Source 16 | 68 | R1-2112720 | CDRX | 16 | 8 | 8 |  | H | 7 | 7 | 50.00% | 9.64% |
| Source 16 | 69 | R1-2112720 | CDRX | 16 | 8 | 16 |  | H | 7 | 7 | 65.00% | 4.10% |
| Source 16 | 70 | R1-2112720 | Genie (CDRX with ideal PDCCH Skipping) | 16 | None | None | Genie is the same for all CDRX | H | 7 | 7 | 90.00% | 73.50% |
| Source 16 | 76 | R1-2112720 | ALWAYS ON | None | None | None |  | H | 7 | 7 | 90.00% | 0.00% |
| Source 16 | 80 | R1-2112720 | ALWAYS ON | None | None | None |  | H | 7 | 7 | 90.00% | 0.00% |

Observations

- In FR2, DL only evaluation, InH, VR/AR30 and low load, it is observed from Source 18 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (10/8/4, 16/14/4) provides the mean power saving gain of 8.17% in the range of 6.28 ~ 10.06% with marginal loss in DL UE satisfied rate.

- In FR2, DL only evaluation, DU, VR/AR30 and low load, it is observed from Source 18 that the R17 PDCCH monitoring adaptation scheme provides the mean power saving gain of 33.80% with marginal loss in DL UE satisfied rate.

Table 63 Source specific data: FR2, DL-only, InH, VR/AR30, low load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 97 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | L | 4 | 8 | 100.00% | - |
| Source 18 | 98 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | L | 4 | 8 | 99.31% | 10.06% |
| Source 18 | 99 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | L | 4 | 8 | 99.31% | 6.28% |
| Source 18 | 101 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | L | 4 | 8 | 100.00% | 33.80% |

Observations

- In FR2, DL only evaluation, InH, VR/AR45 and high load, it is observed from Source 18, Source 15 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (10/8/4, 16/14/4, 10/8/2) provides the mean power saving gain of 7.46% in the range of 5.73 ~ 9.15% with marginal loss in DL UE satisfied rate.

- The choice of a particular R15/16 CDRX configuration (cycle, on duration, and inactivity timer) greatly affects the PS gain.

Observations

- In FR2, DL only evaluation, InH, VR/AR45 and high load, it is observed from Source 18, Source 16 that the R17 PDCCH monitoring adaptation scheme provides the mean power saving gain of 28.58% in the range of 27.36 ~ 29.8% with marginal loss in DL UE satisfied rate.

- In FR2, DL only evaluation, InH, VR/AR45 and high load, it is observed from Source 16 that the R16 cross slot scheduling scheme provides the mean power saving gain of 12.20% with marginal loss in DL UE satisfied rate.

- In FR2, DL only evaluation, InH, VR/AR45 and high load, it is observed from Source 16 that the R17 PDCCH monitoring adaptation + cross slot scheduling scheme provides the mean power saving gain of 30.0% with marginal loss in DL UE satisfied rate.

Table 64 Source specific data: FR2, DL-only, InH, VR/AR45, high load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 115 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 4 | 4 | 94.44% | - |
| Source 18 | 116 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | H | 4 | 4 | 91.67% | 9.15% |
| Source 18 | 117 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | H | 4 | 4 | 93.75% | 5.73% |
| Source 18 | 119 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | H | 4 | 4 | 93.75% | 27.36% |
| Source 15 | 26 | R1-2112572 | R15/16CDRX | 4 | 2 | 2 |  | H | 6 | 6 | 75.56% | 18.00% |
| Source 15 | 27 | R1-2112572 | R15/16CDRX | 8 | 4 | 4 |  | H | 6 | 6 | 9.40% | 15.00% |
| Source 15 | 28 | R1-2112572 | R15/16CDRX | 16 | 8 | 8 |  | H | 6 | 6 | 0.00% | 11.60% |
| Source 15 | 29 | R1-2112572 | R15/16CDRX | 10 | 8 | 2 |  | H | 6 | 6 | 90.00% | 7.50% |
| Source 15 | 30 | R1-2112572 | R15/16CDRX | 10 | 5 | 5 |  | H | 6 | 6 | 3.33% | 13.50% |
| Source 16 | 1 | R1-2112720 | ALWAYS ON | Null | 0 | 0 |  | H | 3 | 3 | 90.00% | 0.00% |
| Source 16 | 2 | R1-2112720 | Cross-slot scheduling | Null | 0 | 0 |  | H | 3 | 3 | 90.00% | 12.20% |
| Source 16 | 3 | R1-2112720 | PDCCH Skipping | Null | 0 | 0 |  | H | 3 | 3 | 90.00% | 29.80% |
| Source 16 | 4 | R1-2112720 | PDCCH Skipping + Cross-slot skipping | Null | 0 | 0 |  | H | 3 | 3 | 90.00% | 30.00% |

Observations

- In FR2, DL only evaluation, InH, VR/AR45 and low load, it is observed from Source 18 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (10/8/4, 16/14/4) provides the mean power saving gain of 7.75% in the range of 5.98 ~ 9.52% with marginal loss in DL UE satisfied rate.

- In FR2, DL only evaluation, InH, VR/AR45 and low load, it is observed from Source 18 that the R17 PDCCH monitoring adaptation scheme provides the mean power saving gain of 28.87% with marginal loss in DL UE satisfied rate.

Table 65 Source specific data: FR2, DL-only, InH, VR45, low load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 109 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | L | 2 | 4 | 100.00% | - |
| Source 18 | 110 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | L | 2 | 4 | 98.61% | 9.52% |
| Source 18 | 111 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | L | 2 | 4 | 98.61% | 5.98% |
| Source 18 | 113 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | L | 2 | 4 | 98.61% | 28.87% |

            - CG

Observations

- In FR2, DL only evaluation, InH, CG30 and high load, it is observed from Source 15, Source 16 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (4/2/2, 8/4/4, 10/8/2, 10/5/5, 16/4/4, 16/8/8, 16/8/16) provides the mean power saving gain of 13.03% in the range of 3.79 ~ 22.66% with marginal loss in DL UE satisfied rate.

- The choice of a particular R15/16 CDRX configuration (cycle, on duration, and inactivity timer) greatly affects the PS gain.

Table 66 Source specific data: FR2, DL-only, InH, CG30, high load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 15 | 16 | R1-2112572 | R15/16CDRX | 4 | 2 | 2 |  | H | 11 | 11 | 98.33% | 18.50% |
| Source 15 | 17 | R1-2112572 | R15/16CDRX | 8 | 4 | 4 |  | H | 11 | 11 | 98.00% | 15.40% |
| Source 15 | 18 | R1-2112572 | R15/16CDRX | 16 | 8 | 8 |  | H | 11 | 11 | 78.10% | 11.60% |
| Source 15 | 19 | R1-2112572 | R15/16CDRX | 10 | 8 | 2 |  | H | 11 | 11 | 98.20% | 7.60% |
| Source 15 | 20 | R1-2112572 | R15/16CDRX | 10 | 5 | 5 |  | H | 11 | 11 | 96.00% | 13.70% |
| Source 16 | 66 | R1-2112720 | ALWAYS ON | None | None | None |  | H | 7 | 7 | 90.00% | 0.00% |
| Source 16 | 67 | R1-2112720 | CDRX | 16 | 4 | 4 |  | H | 7 | 7 | 80.00% | 22.66% |
| Source 16 | 68 | R1-2112720 | CDRX | 16 | 8 | 8 |  | H | 7 | 7 | 90.00% | 9.56% |
| Source 16 | 69 | R1-2112720 | CDRX | 16 | 8 | 16 |  | H | 7 | 7 | 90.00% | 3.79% |

##### UL-only Evaluation

###### DU

Table 67 Summary of FR2, UL-only, power evaluation results for DU

| Scen-arios | App | UL Bit rate (Mbps) | PS scheme, Note 2 | System Load | PSG(%), Note 1,3 |  | source |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Mean (%) | Range (%) |  |
| DU | VR/CG UL Pose | 0.2 | R15/16 CDRX | High | 38.90 | 35.29 ~ 42.51 | Source 18 |
|  | AR UL 1 stream (scene) | 10 | R15/16 CDRX | High | 7.68 | 6.18 ~ 9.18 | Source 18 |
|  |  |  |  | Low | 7.89 | 6.41 ~ 9.36 | Source 18 |
|  |  |  | R17 PDCCH monitoring adaptation | High | 46.21 | 46.21 ~ 51.42 | Source 18 |
|  |  |  |  | Low | 51.43 |  |  |
| Note 1 : PSG was computed for the cases only with marginal loss in % of UL satisfied UE. Note 2: The CDRX configurations considered in each case could be different. The details of considered R15/16 CDRX configurations in this table are listed in the following tables. Note 3: The PSG is computed with respect to power consumption of AlwaysOn scheme. |  |  |  |  |  |  |  |

            - VR/CG

Observation

- In FR2, UL only evaluation, DU, VR/CG pose only and high load, it is observed from Source 18 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (4/2/1,8/3/1) provides the mean power saving gain of 38.90% in the range of 35.29 ~ 42.51% with marginal loss in UL UE satisfied rate.

Table 68 Source specific data: FR2, UL-only, DU, VR/CG Pose only, high load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 187 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 20 | 20 | 96.51% | - |
| Source 18 | 188 | R1-2111046 | R15/16CDRX | 4 | 2 | 1 |  | H | 20 | 20 | 94.13% | 35.29% |
| Source 18 | 189 | R1-2111046 | R15/16CDRX | 8 | 3 | 1 |  | H | 20 | 20 | 92.30% | 42.51% |

No results available for FR2, UL-only, DU, VR/CG Pose only, low load

            - AR with UL 1 stream

Observations

- In FR2, UL only evaluation, DU, AR UL 1 stream, and high load, it is observed from Source 18 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (10/8/4, 16/14/4) provides the mean power saving gain of 7.68% in the range of 6.18 ~ 9.18% with marginal loss in UL UE satisfied rate.

- In FR2, UL only evaluation, DU, AR UL 1 stream and high load, it is observed from Source 18 that the R17 PDCCH monitoring adaptation scheme provides the mean power saving gain of 46.21% with marginal loss in UL UE satisfied rate.

Table 69 Source specific data: FR2, UL-only, DU, AR UL 1 stream, high load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 195 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 8 | 8 | 92.66% | - |
| Source 18 | 196 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | H | 8 | 8 | 91.07% | 9.18% |
| Source 18 | 197 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | H | 8 | 8 | 91.67% | 6.18% |
| Source 18 | 199 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | H | 8 | 8 | 91.27% | 46.21% |

Observations

- In FR2, UL only evaluation, DU, AR UL 1 stream, and low load, it is observed from Source 18 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (10/8/4, 16/14/1) provides the mean power saving gain of 7.89% in the range of 6.41 ~ 9.36% with marginal loss in UL UE satisfied rate.

- In FR2, UL only evaluation, DU, AR UL 1 stream, and low load, it is observed from Source 18 that the R17 PDCCH monitoring adaptation scheme provides the mean power saving gain of 51.43% with marginal loss in DL UE satisfied rate.

Table 70 Source specific data: FR2, UL-only, DU, AR UL 1 stream, low load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 190 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | L | 4 | 8 | 100.00% | - |
| Source 18 | 191 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | L | 4 | 8 | 99.60% | 9.36% |
| Source 18 | 192 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | L | 4 | 8 | 100.00% | 6.41% |
| Source 18 | 194 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | L | 4 | 8 | 100.00% | 51.43% |

No results available for FR2, UL-only, DU, AR 2 streams.

###### InH

Table 71 Summary of FR2, UL-only power evaluation results for InH

| Scen-arios | App | UL Bit rate (Mbps) | PS scheme, Note 2 | System Load | PSG(%), Note 1,3 |  | source |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Mean (%) | Range (%) |  |
| InH | VR/CG UL Pose | 0.2 | R15/16 CDRX | High | 40.53 | 35.99 ~ 45.07 | Source 18 |
|  | AR UL 1 stream (scene) | 10 | R15/16 CDRX | High | 8.16 | 6.58 ~ 9.74 | Source 18 |
|  |  |  |  | Low | 8.60 | 6.96 ~10.24 | Source 18 |
|  |  |  | R17 PDCCH monitoring adaptation | High | 51.32 |  | Source 18 |
|  |  |  |  | Low | 52.35 |  | Source 18 |
| Note 1 : PSG was computed for the cases only with marginal loss in % of UL satisfied UE. Note 2: The CDRX configurations considered in each case could be different. The details of considered R15/16 CDRX configurations in this table are listed in the following tables. Note 3: The PSG is computed with respect to power consumption of AlwaysOn scheme. |  |  |  |  |  |  |  |

            - VR/CG

Observation

- In FR2, UL only evaluation, InH, VR/CG pose only, and high load, it is observed from Source 18 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (4/2/1, 8/3/1) provides the mean power saving gain of 40.53% in the range of 35.99 ~ 45.07% with marginal loss in UL UE satisfied rate.

Table 72 Source specific data: FR2, UL-only, InH, VR/CG Pose only, high load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 174 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 20 | 20 | 97.69% | - |
| Source 18 | 175 | R1-2111046 | R15/16CDRX | 4 | 2 | 1 |  | H | 20 | 20 | 95.90% | 35.99% |
| Source 18 | 176 | R1-2111046 | R15/16CDRX | 8 | 3 | 1 |  | H | 20 | 20 | 92.82% | 45.07% |

No results available for FR2, UL-only, DU, VR/CG Pose only, low load case

            - AR with UL 1 stream

Observations

- In FR2, UL only evaluation, InH, AR UL 1 stream, and high load, it is observed from Source 18 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (10/8/4, 16/14/4) provides the mean power saving gain of 8.16% in the range of 6.58 ~ 9.74% with marginal loss in UL UE satisfied rate.

- In FR2, UL only evaluation, InH, AR UL 1 stream, and high load, it is observed from Source 18 that the R17 PDCCH monitoring adaptation scheme provides the mean power saving gain of 46.21% with marginal loss in UL UE satisfied rate.

Table 73 Source specific data: FR2, UL-only, InH, AR 1 Stream, high load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 182 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | H | 8 | 8 | 95.14% | - |
| Source 18 | 183 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | H | 8 | 8 | 92.71% | 9.74% |
| Source 18 | 184 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | H | 8 | 8 | 94.10% | 6.58% |
| Source 18 | 186 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | H | 8 | 8 | 93.06% | 51.32% |

Observations

- In FR2, UL only evaluation, InH, AR UL 1 stream, and low load, it is observed from Source 18 that the R15/16CDRX scheme with configurations of (cycle/ODT/IAT) = (10/8/4, 16/14/4) provides the mean power saving gain of 8.60% in the range of 6.96 ~ 10.24% with marginal loss in UL UE satisfied rate.

- In FR2, UL only evaluation, InH, AR UL 1 stream, and low load, it is observed from Source 18 that the R17 PDCCH monitoring adaptation scheme provides the mean power saving gain of 52.35% with marginal loss in UL UE satisfied rate.

Table 74 Source specific data:  FR2, UL-only, InH, AR 1 Stream, low load

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 177 | R1-2111046 | AlwaysOn - baseline | 0 | 0 | 0 |  | L | 4 | 8 | 100.00% | - |
| Source 18 | 178 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | L | 4 | 8 | 100.00% | 10.24% |
| Source 18 | 179 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | L | 4 | 8 | 100.00% | 6.96% |
| Source 18 | 181 | R1-2111046 | R17 PDCCH monitoring adaptation | 0 | 0 | 0 |  | L | 4 | 8 | 100.00% | 52.35% |

### Performance Comparison for Parameters/Modelling

#### Trade-off between Capacity and Power

This section captures the CDRX performance evaluation results showing the trade-off between capacity (% of satisfied UE) and power consumption.

Observations

- It is observed from the Source 10, Source 15, Source 18, Source 9, Source 7, Source 11 that there is trade-off relation between % of satisfied UE (or capacity) and power saving gain, that is, in general, high power saving gain can be achieved with the lower % of satisfied UE for CDRX schemes.

Table 75 Source specific data, FR1, DL, DU, VR30

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | Mean PSG of all Ues (%) | % of DL satisfied UE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 10 | 3 | R1-2111521 | R15/16CDRX | 8 | 6 | 6 |  | H | 4 | 5 | 10.80% | 94.00% |
| Source 15 | 39 | R1-2112572 | R15/16CDRX | 10 | 8 | 2 |  | H | 6 | 6 | 9.20% | 93.00% |
| Source 18 | 43 | R1-2111046 | R15/16CDRX | 16 | 14 | 4 |  | H | 13 | 13 | 3.03% | 91.58% |
| Source 9 | 5 | R1-2110811 | R15/16CDRX | 16 | 14 | 4 |  | H | 5 | 5 | 3.46% | 90.67% |
| Source 18 | 42 | R1-2111046 | R15/16CDRX | 10 | 8 | 4 |  | H | 13 | 13 | 4.70% | 90.11% |
| Source 11 | 25 | R1-2111830 | R15/16CDRX | 10 | 8 | 2 |  | H | 3 | 3 | 7.22% | 88.33% |
| Source 9 | 3 | R1-2110811 | R15/16CDRX | 10 | 8 | 4 |  | H | 5 | 5 | 5.53% | 88.29% |
| Source 7 | 16 | R1-2112160 | R15/16CDRX | 10 | 8 | 3 |  | H | 4 | 4 | 4.00% | 84.00% |
| Source 15 | 36 | R1-2112572 | R15/16CDRX | 4 | 2 | 2 |  | H | 6 | 6 | 21.00% | 83.00% |
| Source 10 | 4 | R1-2111521 | R15/16CDRX | 8 | 4 | 6 |  | H | 4 | 5 | 15.70% | 82.75% |
| Source 11 | 23 | R1-2111830 | R15/16CDRX | 4 | 2 | 2 |  | H | 3 | 3 | 16.76% | 76.00% |
| Source 11 | 22 | R1-2111830 | R15/16CDRX | 16 | 12 | 4 |  | H | 3 | 3 | 6.93% | 75.00% |
| Source 9 | 2 | R1-2110811 | R15/16CDRX | 10 | 5 | 4 |  | H | 5 | 5 | 14.68% | 61.05% |
| Source 15 | 37 | R1-2112572 | R15/16CDRX | 8 | 4 | 4 |  | H | 6 | 6 | 18.00% | 61.00% |
| Source 15 | 40 | R1-2112572 | R15/16CDRX | 10 | 5 | 5 |  | H | 6 | 6 | 17.00% | 52.00% |

#### Performance Comparison for Different DL Frame Generation Rates

In this section, we capture the data points showing the relation between DL frame generation rates and UE power consumption.

Observations

- It was observed from Source 16 that increasing application frame generation rate increases UE power consumption.

- In FR1, DL+UL evaluation, DU, AlwaysOn, it was observed from Source 16 that VR 30Mbps with 120fps increases power consumption by 6.45% w.r.t. 60fps case.

Table 76 Source specific data: FR1, DL+UL eval, DU, VR 30Mbps for different DL frame generation rates

| source | data point index | Tdoc source | Power saving scheme | Fps | Load H/L | N1 | C1 | % of DL+UL satisfied UE | Mean PSG of all UEs (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 16 | 5 | R1-2112720 | AlwaysOn - baseline | 60 | H | 11 | 11 | 95.33% | 0.00% |
| Source 16 | 130 | R1-2112720 | AlwaysOn - baseline | 120 | H | 11 | 11 | 98.87% | -6.45% |

#### Performance Comparison for Different Data Rates

In this section, we capture the evaluation results showing the relation between data rates and UE power consumption.

Observations

- It was observed from Source 16 that icreasing application data(bit) rate increases UE power consumption.

- In FR1, DL+UL evaluation, DU, AlwaysOn, it was observed from Source 16 that VR DL bit rate of 45Mbps and 60 Mbps increases power consumption by 2.14 and 4.21% compared to VR DL 30Mbps case.

Table 77 Source specific data: FR1, DL+UL, DU, VR 30Mbps for different data rates

| source | data row index | Tdoc source | Power saving scheme | DL bit rates | Additional Assumptions | Load H/L | N1 | C1 | % of DL+UL satisfied UE | PSG (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 16 | 131 | R1-2112720 | AlwaysOn | 30Mbps |  | L | 1 | 11 | 100% | 0.00% |
| Source 16 | 132 | R1-2112720 | AlwaysOn | 45Mbps |  | L | 1 | 11 | 98.09% | -2.14% |
| Source 16 | 133 | R1-2112720 | AlwaysOn | 60Mbps |  | L | 1 | 11 | 95.71% | -4.21% |

#### Performance Comparison for Different Pose Periodicity

In this section, the impact of different pose periodicities on power consumption is evaluated.

Table 78 Summary of power performance for different periodicity.

| Scen-arios | App | UL Bit rate (Mbps) | UL pose periodicity | PS scheme, Note 2 | PS gain (%), Note 1 |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Mean (%) | Range (%) |  |
| DU | VR/CG UL Pose | 0.2 | 4ms | AlwaysOn | 0 |  | Source 16 |
|  |  | 0.1 | 8ms | AlwaysOn | 2.27 |  | Source 16 |
|  |  | 0.048 | 16.67ms | AlwaysOn | 10.83 |  | Source 16 |
| Note 1 : PSG was computed for the cases only with marginal loss in % of UL satisfied UE. Note 2: The CDRX configurations considered in each case could be different. The details of considered R15/16 CDRX configurations in this table are listed in the following tables. |  |  |  |  |  |  |  |

Observations

- It was observed from Source 16 that reducing pose periodicity could decrease power consumption.

- In FR1, DL+UL evaluation, DU, Pose only, AlwaysOn, it was observed from Source 16 the pose tx with periodicity of 8ms (or 125Hz) has power saving gain of 2.27% w.r.t AlwaysOn with periodicity of 4ms.

- In FR1, DL+UL evaluation, DU, Pose only, AlwaysOn, it was observed from Source 16 the pose tx with periodicity of 16.67ms (or 60Hz) has power saving gain of 10.83% w.r.t AlwaysOn with periodicity of 4ms.

Table 79 Source specific data: FR1, DU, DL+UL, VR30, UL pose

| source | data row index | Tdoc source | Power saving scheme | Pose Periodicity | Load H/L | N1 | C1 | % of DL+UL satisfied UE | Mean PSG of all UEs (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 16 | 5 | R1-2112720 | AlwaysOn | 4ms | H | 11 | 11 | 95.49% | 0.00% |
| Source 16 | 134 | R1-2112720 | AlwaysOn | 8ms | H | 11 | 11 | 95.15% | 2.27% |
| Source 16 | 135 | R1-2112720 | AlwaysOn | 16.67ms | H | 11 | 11 | 95.75% | 10.83% |

### Potential Enhancements

There have been no RAN1 discussion on aligning the implementation details of the proposed enhancement schemes presented in this section, or aligning the evaluation methodologies to comprehensively model them. The simulation results presented in this section are primarily results from individual sources that may have certain discrepancies in the details of the proposed enhancement scheme and/or additional assumptions made for evaluation purposes.

#### Enhanced CDRX

In this section, we provide performance evaluation results of eCDRX where eCDRX is a set of enhanced CDRX mechanisms which allow adjusting DRX On duration start time offset to be aligned with each XR DL traffic arrival time, or configure a CDRX cycle pattern with different cycle values instead of only one CDRX cycle, etc.

##### FR1

###### DL+UL joint evaluation

Table 80 Summary of FR1, DL+UL power evaluation results for eCDRX

| Scen-arios | App | DL Bit rate (Mbps) | PS scheme, Note 2 | PSG (%), Note 1,4 |  | Source |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Mean (%) | Range (%) |  |
| DU | VR | 30 | eCDRX | 11.64 | 4.51 ~ 23.49 | Source 18, Source 7, Source 16 |
|  | CG | 30 | eCDRX | 6 |  | Source 7 |
|  | AR (UL 1/2 streams) | 30 | eCDRX | 11.06 | 4.6 ~ 20.77 | Source 18 |
| InH | VR | 30 | eCDRX | 17.63 | 7.23 ~ 25.12 | Source 20, Source 18 |
|  |  | 45 | eCDRX | 25.64 | 25.63 ~ 25.65 | Source 20 |
|  | CG | 30 | eCDRX | 18.25 | 18.23 ~ 18.26 | Source 20 |
|  | AR (UL 1/2 streams) | 30 | eCDRX | 12.23 | 4.82 ~ 23.61% | Source 18 |
| Note 1 : PSG was computed for the cases only with marginal loss in % of DL+UL satisfied UE. Note 2: The CDRX configurations considered in each case could be different. The details of considered eCDRX configurations in this table are listed in the following tables. Note 3: For comparison with R15/16 CDRX results, see sections 9.3.1 including baseline performance evaluation results. Note 4: The PSG is computed with respect to power consumption of AlwaysOn scheme. |  |  |  |  |  |  |

            - DU

Observation

- In FR1, DL+UL evaluation, DU, VR30, it was observed from Source 18, Source 7, Source 16 that the enhanced CDRX scheme provides the mean power saving gain of 11.64% in the range of 4.51 ~ 23.49% with marginal loss in DL+UL UE satisfied rate.

Table 81 Source specific data: eCDRX, FR1, DL+UL, DU, VR30

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | % of UL satisfied UE | % of DL + UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 239 | R1-2111046 | Note 1 | 16 | 10 | 4 |  | L | 7 | 13 |  |  | 100.00% | 9.09% |
| Source 18 | 240 | R1-2111046 | Note 2 | 16 | 6 | 4 |  | L | 7 | 13 |  |  | 100.00% | 23.49% |
| Source 18 | 247 | R1-2111046 | Note 1 | 16 | 10 | 4 |  | H | 13 | 13 |  |  | 92.06% | 7.05% |
| Source 18 | 248 | R1-2111046 | Noe 2 | 16 | 6 | 4 |  | H | 13 | 13 |  |  | 91.21% | 21.93% |
| Source 7 | 13 | R1-2112160 | eCDRX | 16.6666 | 13 | 0 |  | H | 4 | 4 |  |  | 85.00% | 6.00% |
| Source 16 | 56 | R1-2112720 | eCDRX | 16/17/17 | 10 | 10 |  | H | 11 | 11 | 97.66% | 84.85% | 82.86% | 9.43% |
| Source 16 | 57 | R1-2112720 | eCDRX | 16/17/17 | 12 | 12 |  | H | 11 | 11 | 97.58% | 96.62% | 94.20% | 4.51% |
| Note 1. e-CDRX adapting to the lower boundary of jitter Note 2. e-CDRX adapting to quasi (ideal)-period position |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Observation

- In FR1, DL+UL evaluation, DU, CG30, it was observed from Source 7 that the enhanced CDRX scheme provides the mean power saving gain of 6.0% with marginal loss in DL+UL UE satisfied rate.

Table 82 Source specific data: eCDRX, FR1, DL+UL, DU, CG30

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load: H/L | N1 | C1 | % of DL satisfied UE | % of UL satisfied UE | % of DL+UL satisfied UE | Mean PSG of all UEs (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 7 | 4 | R1-2112160 | eCDRX | 16.6666 | 13 | 0 |  | H | 4 | 4 |  |  | 87.00% | 6.00% |

Observation

- In FR1, DL+UL evaluation, DU, AR30, it was observed from Source 18 that the enhanced CDRX scheme provides the mean power saving gain of 11.06% in the rage of 4.60 ~ 20.77% with marginal loss in DL+UL UE satisfied rate.

Table 83 Source specific data: eCDRX, FR1, DL+UL, DU, AR30

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | % of UL satisfied UE | % of DL + UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 271 | R1-2111046 | Note 3 | 16 | 10 | 4 | Note 1 | L | 5 | 9 |  |  | 96.19% | 9.60% |
| Source 18 | 272 | R1-2111046 | Note 4 | 16 | 6 | 4 | Note 1 | L | 5 | 9 |  |  | 95.87% | 20.77% |
| Source 18 | 279 | R1-2111046 | Note 3 | 16 | 10 | 4 | Note 1 | H | 9 | 9 |  |  | 92.06% | 6.66% |
| Source 18 | 280 | R1-2111046 | Note 4 | 16 | 6 | 4 | Note 1 | H | 9 | 9 |  |  | 90.83% | 14.04% |
| Source 18 | 303 | R1-2111046 | Note 3 | 16 | 10 | 4 | Note 2 | L | 4 | 7 |  |  | 100.00% | 5.28% |
| Source 18 | 304 | R1-2111046 | Note 4 | 16 | 6 | 4 | Note 2 | L | 4 | 7 |  |  | 100.00% | 14.34% |
| Source 18 | 311 | R1-2111046 | Note 3 | 16 | 10 | 4 | Note 2 | H | 7 | 7 |  |  | 91.38% | 4.60% |
| Source 18 | 312 | R1-2111046 | Note 4 | 16 | 6 | 4 | Note 2 | H | 7 | 7 |  |  | 90.48% | 13.19% |
| Note 1 AR with single UL stream. Note 2 AR with two UL streams. Note 3 e-CDRX adapting to the lower boundary of jitter Note 4 e-CDRX adapting to quasi (ideal)-period position |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

            - InH

Observation

- In FR1, DL+UL evaluation, InH, VR30, it was observed from Source 20, Source 18 that the enhanced CDRX scheme provides the mean power saving gain of 17.63% in the range of 7.23 ~ 25.12% with marginal loss in DL+UL UE satisfied rate.

Table 84 Source specific data: eCDRX, FR1, DL+UL, InH, VR30

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | % of UL satisfied UE | % of DL + UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 223 | R1-2111046 | Note 5 | 16 | 10 | 4 |  | L | 5 | 10 |  |  | 100.00% | 9.38% |
| Source 18 | 224 | R1-2111046 | Note 6 | 16 | 6 | 4 |  | L | 5 | 10 |  |  | 100.00% | 25.12% |
| Source 18 | 231 | R1-2111046 | Note 5 | 16 | 10 | 4 |  | H | 10 | 10 |  |  | 92.06% | 7.23% |
| Source 18 | 232 | R1-2111046 | Note 6 | 16 | 6 | 4 |  | H | 10 | 10 |  |  | 90.70% | 23.56% |
| Source 20 | 7 | R1-2111351 | Note 4 | 16 | 8 | 4 | Note 1,2 | H | 11 | 11 | 86.36% | 100.00% | 86.36% | 20.24% |
| Source 20 | 8 | R1-2111351 | Note 4 | 16 | 8 | 4 | Note 1,3 | H | 11 | 11 | 86.36% | 100.00% | 86.36% | 20.22% |
| Note 1. DL and UL were simulated separately and collected traces are combined as a single timeline for DL+UL joint power evaluation. Note 2. Option 2(Linear interpolation in linear domain) for UL power model Note 3. Option 1(two-step Qauntization) for UL power model Note 4. eCDRX(change drx-startoffset per 100ms) Note 5. e-CDRX adapting to the lower boundary of jitter Note 6. e-CDRX adapting to quasi (ideal)-period position |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Observation

- In FR1, DL+UL evaluation, InH, VR45, it was observed that Source 20 that the enhanced CDRX scheme provides the mean power saving gain of 25.64% in the range of 25.63 ~ 25.65% with marginal loss in DL+UL UE satisfied rate.

Table 85 Source specific data: eCDRX, FR1, DL+UL, InH, VR45

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | % of UL satisfied UE | % of DL + UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 20 | 15 | R1-2111351 | Note 4 | 16 | 8 | 4 | Note 1,2 | H | 7 | 7 | 80.00% | 100.00% | 80.00% | 25.65% |
| Source 20 | 16 | R1-2111351 | Note 4 | 16 | 8 | 4 | Note 1,3 | H | 7 | 7 | 80.00% | 100.00% | 80.00% | 25.63% |
| Note 1. DL and UL were simulated separately and collected traces are combined as a single timeline for DL+UL joint power evaluation. Note 2. Option 2(Linear interpolation in linear domain) for UL power model Note 3. Option 1(two-step Qauntization) for UL power model Note 4. eCDRX(change drx-startoffset per 100ms) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Observation

- In FR1, DL+UL evaluation, InH, CG30, it was observed from Source 20 that the enhanced CDRX scheme provides the mean power saving gain of 18.25% in the range of 18.23 ~ 18.26% with marginal loss in DL+UL UE satisfied rate.

Table 86 Source specific data: eCDRX, FR1, DL+UL, InH, CG30

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | % of UL satisfied UE | % of DL + UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 20 | 23 | R1-2111351 | Note 4 | 16 | 8 | 4 | Note 1,2 | H | 12 | 12 | 85.40% | 100.00% | 85.40% | 18.26% |
| Source 20 | 24 | R1-2111351 | Note 4 | 16 | 8 | 4 | Note 1,3 | H | 12 | 12 | 85.40% | 100.00% | 85.40% | 18.23% |
| Note 1. DL and UL were simulated separately and merged for DL+UL joint power evaluation. Note 2. Option 2(Linear interpolation in linear domain) for UL power model Note 3. Option 1(two-step Qauntization) for UL power model Note 4. eCDRX(change drx-startoffset per 100ms) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Observation

- In FR1, DL+UL evaluation, InH, AR30, it was observed from Source 18 that the enhanced CDRX scheme provides the mean power saving gain of 12.23% in the range of 4.82 ~ 23.61% with marginal loss in DL+UL UE satisfied rate.

Table 87 Source specific data: eCDRX, FR1, DL+UL, InH, AR30 (1 & 2 streams)

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | % of UL satisfied UE | % of DL + UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 255 | R1-2111046 | Note 1 | 16 | 10 | 4 | Note 3 | L | 5 | 10 |  |  | 100.00% | 10.76% |
| Source 18 | 256 | R1-2111046 | Note 2 | 16 | 6 | 4 | Note 3 | L | 5 | 10 |  |  | 100.00% | 23.61% |
| Source 18 | 263 | R1-2111046 | Note 1 | 16 | 10 | 4 | Note 3 | H | 10 | 10 |  |  | 91.90% | 6.95% |
| Source 18 | 264 | R1-2111046 | Note 2 | 16 | 6 | 4 | Note 3 | H | 10 | 10 |  |  | 90.83% | 14.77% |
| Source 18 | 287 | R1-2111046 | Note 1 | 16 | 10 | 4 | Note 4 | L | 5 | 10 |  |  | 100.00% | 6.29% |
| Source 18 | 288 | R1-2111046 | Note 2 | 16 | 6 | 4 | Note 4 | L | 5 | 10 |  |  | 100.00% | 16.65% |
| Source 18 | 295 | R1-2111046 | Note 1 | 16 | 10 | 4 | Note 4 | H | 10 | 10 |  |  | 91.59% | 4.82% |
| Source 18 | 296 | R1-2111046 | Note 2 | 16 | 6 | 4 | Note 4 | H | 10 | 10 |  |  | 90.56% | 13.96% |
| Note 1. e-CDRX adapting to the lower boundary of jitter Note 2. e-CDRX adapting to quasi (ideal)-period position Note 3. AR UL 1 stream Note 4. AR UL 2 streams |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

            - UMa

No results available for UMa

###### DL-only Evaluation

Table 88 Summary of FR1, DL-only power evaluation results for eCDRX

| Scen-arios | App | DL Bit rate (Mbps) | PS scheme, Note 2 | PSG(%), Note 1,4 |  | Source |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Mean (%) | Range (%) |  |
| DU | VR | 30 | eCDRX | 15.70 | 5.76 ~ 34.95 | Source 9, Source 18, Source 7, Source 16 |
|  |  | 45 | eCDRX | 18.14 | 9.72 ~ 27.26 | Source 18 |
| InH | VR | 30 | eCDRX | 20.812 | 9.36 ~ 29.43 | Source 20, Source 18 |
|  |  | 45 | eCDRX | 19.96 | 9.42 ~ 29.1 | Source 20, Source 18 |
|  | CG | 30 | eCDRX | 26.38 | 26.38 | Source 20 |
| UMa | VR | 30 | eCDRX | 18.88 | 10.05 ~ 29.06 | Source 18 |
|  |  | 45 | eCDRX | 18.22 | 9.86 ~ 27.33 | Source 18 |
| Note 1 : PSG was computed for the cases only with marginal loss in % of DL satisfied UE. Note 2: The CDRX configurations considered in each case could be different. The details of considered eCDRX configurations in this table are listed in the following tables. Note 3: For comparison with R15/16 CDRX results, see section 9.3.1 including baseline performance evaluation results. Note 4: The PSG is computed with respect to power consumption of AlwaysOn scheme. |  |  |  |  |  |  |

            - DU

Observation

- In FR1, DL only evaluation, DU, VR30, it was observed from Source 9, Source 18, Source 7, Source 16 that the enhanced CDRX scheme provides the mean power saving gain of 15.70% in the range of 5.76 ~ 34.95% with marginal loss in DL UE satisfied rate.

Table 89 Source specific data: eCDRX, FR1, DL-only, DU, VR30

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 9 | 6 | R1-2110811 | eCDRX | 17/17/16 | 8 | 4 | OnDurationStartOffset=-3ms | H | 5 | 5 | 80.29% | 17.84% |
| Source 9 | 7 | R1-2110811 | eCDRX | 17/17/16 | 8 | 4 | OnDurationStartOffset=-2ms | H | 5 | 5 | 86.10% | 19.29% |
| Source 9 | 8 | R1-2110811 | eCDRX | 17/17/16 | 10 | 4 | OnDurationStartOffset=-4ms | H | 5 | 5 | 86.86% | 13.00% |
| Source 9 | 9 | R1-2110811 | eCDRX | 17/17/16 | 8 | 8 | OnDurationStartOffset=-3ms | H | 5 | 5 | 85.43% | 7.93% |
| Source 9 | 10 | R1-2110811 | eCDRX | 17/17/16 | 8 | 8 | OnDurationStartOffset=-2ms | H | 5 | 5 | 90.95% | 9.77% |
| Source 9 | 11 | R1-2110811 | eCDRX | 17/17/16 | 10 | 8 | OnDurationStartOffset=-4ms | H | 5 | 5 | 91.62% | 5.76% |
| Source 9 | 21 | R1-2110811 | eCDRX | 17/17/16 | 8 | 4 | OnDurationStartOffset=-3ms | L | 3 | 5 | 93.65% | 18.65% |
| Source 9 | 22 | R1-2110811 | eCDRX | 17/17/16 | 8 | 4 | OnDurationStartOffset=-2ms | L | 3 | 5 | 96.51% | 20.17% |
| Source 9 | 23 | R1-2110811 | eCDRX | 17/17/16 | 10 | 4 | OnDurationStartOffset=-4ms | L | 3 | 5 | 97.62% | 13.63% |
| Source 9 | 24 | R1-2110811 | eCDRX | 17/17/16 | 8 | 8 | OnDurationStartOffset=-3ms | L | 3 | 5 | 95.40% | 8.46% |
| Source 9 | 25 | R1-2110811 | eCDRX | 17/17/16 | 8 | 8 | OnDurationStartOffset=-2ms | L | 3 | 5 | 98.10% | 10.52% |
| Source 9 | 26 | R1-2110811 | eCDRX | 17/17/16 | 10 | 8 | OnDurationStartOffset=-4ms | L | 3 | 5 | 98.41% | 6.26% |
| Source 18 | 36 | R1-2111046 | Note 1 | 16 | 10 | 4 | 0 | L | 7 | 13 | 100.00% | 12.49% |
| Source 18 | 37 | R1-2111046 | Note 2 | 16 | 6 | 4 | 0 | L | 7 | 13 | 100.00% | 27.49% |
| Source 18 | 44 | R1-2111046 | Note 1 | 16 | 10 | 4 | 0 | H | 13 | 13 | 91.70% | 8.67% |
| Source 18 | 45 | R1-2111046 | Note 2 | 16 | 6 | 4 | 0 | H | 13 | 13 | 91.21% | 21.72% |
| Source 7 | 18 | R1-2112160 | eCDRX | 16.6666 | 8 | 3 | 0 | H | 4 | 4 | 84.00% | 22.00% |
| Source 16 | 61 | R1-2112720 | eCDRX | 16/17/17 | 4 | 6 | 0 | H | 11 | 11 | 95.76% | 34.95% |
| Source 16 | 62 | R1-2112720 | eCDRX | 16/17/17 | 6 | 6 | 0 | H | 11 | 11 | 96.45% | 28.01% |
| Source 16 | 63 | R1-2112720 | eCDRX | 16/17/17 | 8 | 8 | 0 | H | 11 | 11 | 96.79% | 19.98% |
| Source 16 | 64 | R1-2112720 | eCDRX | 16/17/17 | 10 | 10 | 0 | H | 11 | 11 | 96.19% | 12.19% |
| Source 16 | 65 | R1-2112720 | eCDRX | 16/17/17 | 12 | 12 | 0 | H | 11 | 11 | 96.80% | 6.66% |
| Note 1. e-CDRX adapting to the lower boundary of jitter Note 2. e-CDRX adapting to quasi (ideal)-period position |  |  |  |  |  |  |  |  |  |  |  |  |

Observation

- In FR1, DL only evaluation, DU, VR45, it was observed from Source 18 that the enhanced CDRX scheme provides the mean power saving gain of 18.14% in the range of 9.72 ~ 27.26% with marginal loss in DL UE satisfied rate.

Table 90 Source specific data: eCDRX, FR1, DL-only, DU, VR45

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 52 | R1-2111046 | Note 1 | 16 | 10 | 4 | 0 | L | 3 | 6 | 98.94% | 12.61% |
| Source 18 | 53 | R1-2111046 | Note 2 | 16 | 6 | 4 | 0 | L | 3 | 6 | 99.47% | 27.26% |
| Source 18 | 60 | R1-2111046 | Note 1 | 16 | 10 | 4 | 0 | H | 6 | 6 | 95.63% | 9.72% |
| Source 18 | 61 | R1-2111046 | Note 2 | 16 | 6 | 4 | 0 | H | 6 | 6 | 94.18% | 22.95% |
| Note 1. e-CDRX adapting to the lower boundary of jitter Note 2. e-CDRX adapting to quasi (ideal)-period position |  |  |  |  |  |  |  |  |  |  |  |  |

            - InH

Observation

- In FR1, DL only evaluation, InH, VR30, it was observed from Source 18, Source 20 that the enhanced CDRX scheme provides the mean power saving gain of 20.81% in the range of 9.36 ~ 29.43% with marginal loss in DL UE satisfied rate.

Table 91 Source specific data: eCDRX, FR1, DL-only, InH, VR30

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 4 | R1-2111046 | Note 1 | 16 | 10 | 4 | 0 | L | 5 | 10 | 100.00% | 13.05% |
| Source 18 | 5 | R1-2111046 | Note 2 | 16 | 6 | 4 | 0 | L | 5 | 10 | 100.00% | 28.38% |
| Source 18 | 12 | R1-2111046 | Note 1 | 16 | 10 | 4 | 0 | H | 10 | 10 | 91.94% | 9.36% |
| Source 18 | 13 | R1-2111046 | Note 2 | 16 | 6 | 4 | 0 | H | 10 | 10 | 91.25% | 23.84% |
| Source 20 | 30 | R1-2111351 | Note 3 | 16 | 8 | 4 |  | H | 11 | 11 | 86.36% | 29.43% |
| Note 1. e-CDRX adapting to the lower boundary of jitter Note 2. e-CDRX adapting to quasi (ideal)-period position Note 3. eCDRX(change drx-startoffset per 100ms) |  |  |  |  |  |  |  |  |  |  |  |  |

Observation

- In FR1, DL only evaluation, InH, VR45, it was observed from Source 18, Source 20 that the enhanced CDRX scheme provides the mean power saving gain of 19.96% in the range of 9.42 ~ 29.1% with marginal loss in DL UE satisfied rate.

Table 92 Source specific data: eCDRX, FR1, DL-only, InH, VR45

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 20 | R1-2111046 | Note 1 | 16 | 10 | 4 | 0 | L | 3 | 5 | 100.00% | 11.96% |
| Source 18 | 21 | R1-2111046 | Note 2 | 16 | 6 | 4 | 0 | L | 3 | 5 | 100.00% | 26.74% |
| Source 18 | 28 | R1-2111046 | Note 1 | 16 | 10 | 4 | 0 | H | 5 | 5 | 96.67% | 9.42% |
| Source 18 | 29 | R1-2111046 | Note 3 | 16 | 6 | 4 | 0 | H | 5 | 5 | 93.89% | 22.61% |
| Source 20 | 34 | R1-2111351 | eCDRX(change drx-startoffset per 100ms) | 16 | 8 | 4 | 0 | H | 7 | 7 | 80% | 29.1% |
| Note 1. e-CDRX adapting to the lower boundary of jitter Note 2. e-CDRX adapting to quasi (ideal) - period position Note 3. e-CDRX adapting to quasi (ideal) - period position |  |  |  |  |  |  |  |  |  |  |  |  |

Observations

- In FR1, DL only evaluation, InH, CG30, it was observed from Source 20 that the enhanced CDRX scheme provides the mean power saving gain of 26.38% with marginal loss in DL UE satisfied rate.

Table 93 Source specific data: eCDRX, FR1, DL-only, InH, CG30

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 20 | 38 | R1-2111351 | eCDRX(change drx-startoffset per 100ms) | 16 | 8 | 4 | 0 | H | 12 | 12 | 85.40% | 26.38% |

            - UMa

Observations

- In FR1, DL only evaluation, UMa, VR30, it was observed from Source 18 that the enhanced CDRX scheme provides the mean power saving gain of 18.88% in the range of 10.05 ~ 29.06 % with marginal loss in DL UE satisfied rate.

Table 94 Source specific data: eCDRX, FR1, DL-only, UMa, VR30

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 68 | R1-2111046 | e-CDRX adapting to the lower boundary of jitter | 16 | 10 | 4 | 0 | L | 4 | 8 | 98.81% | 13.09% |
| Source 18 | 69 | R1-2111046 | e-CDRX adapting to quasi (ideal)-period position | 16 | 6 | 4 | 0 | L | 4 | 8 | 97.22% | 29.06% |
| Source 18 | 76 | R1-2111046 | e-CDRX adapting to the lower boundary of jitter | 16 | 10 | 4 | 0 | H | 8 | 8 | 93.35% | 10.05% |
| Source 18 | 77 | R1-2111046 | e-CDRX adapting to quasi (ideal)-period position | 16 | 6 | 4 | 0 | H | 8 | 8 | 91.87% | 23.33% |

Observation

- In FR1, DL only evaluation, UMa, VR45, it was observed from Source 18 that the enhanced CDRX scheme provides the mean power saving gain of 18.22% in the range of 9.86 ~ 27.33% with marginal loss in DL UE satisfied rate.

Table 95 Source specific data: eCDRX, FR1, DL-only, UMa, VR45

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 84 | R1-2111046 | e-CDRX adapting to the lower boundary of jitter | 16 | 10 | 4 | 0 | L | 2 | 4 | 96.83% | 12.09% |
| Source 18 | 85 | R1-2111046 | e-CDRX adapting to quasi (ideal)-period position | 16 | 6 | 4 | 0 | L | 2 | 4 | 96.83% | 27.33% |
| Source 18 | 92 | R1-2111046 | e-CDRX adapting to the lower boundary of jitter | 16 | 10 | 4 | 0 | H | 4 | 4 | 94.05% | 9.86% |
| Source 18 | 93 | R1-2111046 | e-CDRX adapting to quasi (ideal)-period position | 16 | 6 | 4 | 0 | H | 4 | 4 | 91.67% | 23.59% |

###### UL-only Evaluation

Table 96 Summary of FR1, UL-only power evaluation results for eCDRX

| Scen-arios | App | DL Bit rate (Mbps) | PS scheme, Note 2 | PS Gain (%), Note 1,4 |  | source |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Mean (%) | Range (%) |  |
| DU | AR UL 1 / 2 streams | 10.2 | eCDRX | 25.56% | 19.89 ~ 32.02% | Source 18 |
| InH | AR UL 1 / 2 streams | 10.2 | eCDRX | 28.67% | 22.66 ~ 35.24% | Source 18 |
| Note 1 : PSG was computed for the cases only with marginal loss in % of UL satisfied UE. Note 2: The CDRX configurations considered in each case could be different. The details of considered eCDRX configurations in this table are listed in the following tables. Note 3: For comparison with R15/16 CDRX results, see section 9.3.1 including baseline performance evaluation results. Note 4: The PSG is computed with respect to power consumption of AlwaysOn scheme. |  |  |  |  |  |  |

            - DU

No results are available for FR1, UL-only, DU, VR/CG Pose only

Observation

- In FR1, UL only evaluation, DU, AR UL 1 & 2 streams, it was observed from Source 18 that the enhanced CDRX scheme provides the mean power saving gain of 25.56% in the range of 19.89 ~ 32.02% with marginal loss in UL UE satisfied rate.

Table 97 Source specific data: eCDRX, FR1, UL-only, DU, AR UL 1 & 2 stream

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 164 | R1-2111046 | Note 1 | 16 | 6 | 4 | AR UL 1 stream | L | 5 | 9 | 95.56% | 32.02% |
| Source 18 | 169 | R1-2111046 | Note 1 | 16 | 6 | 4 | AR UL 1 stream | H | 9 | 9 | 91.60% | 28.99% |
| Source 18 | 213 | R1-2111046 | Note 1 | 16 | 6 | 4 | AR UL 2 streams | L | 4 | 7 | 100.00% | 21.35% |
| Source 18 | 218 | R1-2111046 | Note 1 | 16 | 6 | 4 | AR UL 2 streams | H | 7 | 7 | 90.48% | 19.89% |
| Note 1. e-CDRX adapting to quasi (ideal)-period position |  |  |  |  |  |  |  |  |  |  |  |  |

            - InH

Note results available for FR1, UL-only, InH, VR/CG Pose only.

Observation

- In FR1, UL only evaluation, DU, AR UL 1 & 2 streams, it was observed from Source 18 that the enhanced CDRX scheme provides the mean power saving gain of 28.67% in the range of 23.66 ~ 35.24% with marginal loss in UL UE satisfied rate.

Table 98 Source specific data: eCDRX, FR1, UL-only, InH, AR UL 1 & 2 streams

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of UL satisfied UE | Mean PSG of all UEs (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 151 | R1-2111046 | Note 1 | 16 | 6 | 4 | AR UL 1 stream | L | 7 | 13 | 100% | 35.24% |
| Source 18 | 156 | R1-2111046 | Note 1 | 16 | 6 | 4 | AR UL 1 stream | H | 13 | 13 | 92.38% | 33.64% |
| Source 18 | 203 | R1-2111046 | Note 1 | 16 | 6 | 4 | AR UL 2 stream | L | 6 | 12 | 100% | 23.66% |
| Source 18 | 208 | R1-2111046 | Note 1 | 16 | 6 | 4 | AR UL 2 stream | H | 12 | 12 | 91.90% | 22.17% |
| Note 1. e-CDRX adapting to quasi (ideal)-period position |  |  |  |  |  |  |  |  |  |  |  |  |

            - UMa

No results available for UMa

##### FR2

###### DL-only evaluation

Table 99 Summary of FR2, DL-only power evaluation results for eCDRX

| Scen-arios | App | DL Bit rate (Mbps) | PS scheme, Note 2 | PS Gain (%), Note 1,4 |  | source |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Mean (%) | Range (%) |  |
| DU | VR | 30 | eCDRX | 31.97% | 31.30 ~ 32.63% | Source 18 |
|  |  | 45 | eCDRX | 27.87% | 27.16 ~ 28.57% | Source 18 |
| InH | VR | 30 | eCDRX | 16.42% | 0.3 ~ 34.89% | Source 18, Source 16 |
|  |  | 45 | eCDRX | 28.81% | 28.37 ~ 29.25% | Source 18 |
| Note 1 : PSG was computed for the cases only with marginal loss in % of DL satisfied UE. Note 2: The CDRX configurations considered in each case could be different. The details of considered eCDRX configurations in this table are listed in the following tables. Note 3: For comparison with R15/16 CDRX results, see section 9.3.1 including baseline performance evaluation results. Note 4: The PSG is computed with respect to power consumption of AlwaysOn scheme. |  |  |  |  |  |  |

            - DU

Observation

- In FR2, DL only evaluation, DU, VR30, it was observed from Source 18 that the enhanced CDRX scheme provides the mean power saving gain of 31.97% in the range of 31.30 ~ 32.63% with marginal loss in DL UE satisfied rate.

Table 100 Source specific data: eCDRX, FR2, DL-only, DU, VR30

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all UEs (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 124 | R1-2111046 | e-CDRX adapting to quasi (ideal)-period position | 16 | 8 | 4 |  | L | 7 | 13 | 99.09% | 32.63% |
| Source 18 | 130 | R1-2111046 | e-CDRX adapting to quasi (ideal)-period position | 16 | 8 | 4 |  | H | 13 | 13 | 91.97% | 31.30% |

Observation

- In FR2, DL only evaluation, DU, VR45, it was observed from Source 18 that the enhanced CDRX scheme provides the mean power saving gain of 27.87% in the range of 27.16 ~ 28.57% with marginal loss in DL UE satisfied rate.

Table 101 Source specific data: eCDRX, FR2, DL-only, DU, VR45

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all UEs (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 136 | R1-2111046 | e-CDRX adapting to quasi (ideal)-period position | 16 | 8 | 4 |  | L | 4 | 8 | 100.00% | 28.57% |
| Source 18 | 142 | R1-2111046 | e-CDRX adapting to quasi (ideal)-period position | 16 | 8 | 4 |  | H | 8 | 8 | 91.47% | 27.16% |

            - InH

Observation

- In FR2, DL only evaluation, InH, VR30, it was observed from Source 18, Source 16 that the enhanced CDRX scheme provides the mean power saving gain of 16.42% in the range of 0.3 ~ 34.89% with marginal loss in DL UE satisfied rate.

Table 102 Source specific data: eCDRX, FR2, DL-only, InH, VR30

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 100 | R1-2111046 | e-CDRX adapting to quasi (ideal)- period position | 16 | 8 | 4 |  | L | 4 | 8 | 98.61% | 34.89% |
| Source 18 | 106 | R1-2111046 | e-CDRX adapting to quasi (ideal)- period position | 16 | 8 | 4 |  | H | 8 | 8 | 90.97% | 33.68% |
| Source 16 | 77 | R1-2112720 | eCDRX | 16/16/15 | 4 | 4 |  | H | 7 | 7 | 90.00% | 18.93% |
| Source 16 | 78 | R1-2112720 | eCDRX | 16/16/15 | 8 | 8 |  | H | 7 | 7 | 90.00% | 7.71% |
| Source 16 | 79 | R1-2112720 | eCDRX | 16/16/15 | 8 | 16 |  | H | 7 | 7 | 90.00% | 0.30% |
| Source 16 | 81 | R1-2112720 | eCDRX | 16/16/15 | 4 | 4 |  | H | 7 | 7 | 27.00% | 25.10% |
| Source 16 | 82 | R1-2112720 | eCDRX | 16/16/15 | 8 | 8 |  | H | 7 | 7 | 84.00% | 8.28% |
| Source 16 | 83 | R1-2112720 | eCDRX | 16/16/15 | 8 | 16 |  | H | 7 | 7 | 88.00% | 2.43% |

Observation

- In FR2, DL only evaluation, InH, VR45, it was observed from Source 18 that the enhanced CDRX scheme provides the mean power saving gain of 28.81% in the range of 28.37 ~ 29.25% with marginal loss in DL UE satisfied rate.

Table 103 Source specific data: eCDRX, FR2, DL-only, InH, VR45

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all UEs (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 112 | R1-2112720 | e-CDRX adapting to quasi (ideal)- period position | 16 | 8 | 4 |  | L | 2 | 4 | 100.00% | 29.25% |
| Source 18 | 118 | R1-2112720 | e-CDRX adapting to quasi (ideal)- period position | 16 | 8 | 4 |  | H | 4 | 4 | 91.67% | 28.37% |

###### UL-only evaluation

Table 104 Summary of FR2, UL-only power evaluation results for eCDRX

| Scen-arios | App | DL Bit rate (Mbps) | PS scheme, Note 2 | PSG(%), Note 1,4 |  | source |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Mean (%) | Range (%) |  |
| DU | AR UL 1 stream | 10 | eCDRX | 32.35 | 31.72 ~ 32.97 | Source 18 |
| InH | AR UL 1 stream | 10 | eCDRX | 37.57% | 36.79 ~ 38.35 | Source 18 |
| Note 1 : PSG was computed for the cases only with marginal loss in % of UL satisfied UE. Note 2: The CDRX configurations considered in each case could be different. The details of considered eCDRX configurations in this table are listed in the following tables. Note 3: For comparison with R15/16 CDRX results, see section 9.3.1 including baseline performance evaluation results. Note 4: The PSG is computed with respect to power consumption of AlwaysOn scheme. |  |  |  |  |  |  |

            - DU

Observation

- In FR2, UL only evaluation, DU, AR UL 1 stream, it was observed from Source 18 that the enhanced CDRX scheme provides the mean power saving gain of 32.35% in the range of 31.72 ~ 32.97% with marginal loss in UL UE satisfied rate.

Table 105 Source specific data: eCDRX, FR2, UL-only, DU, AR UL 1 stream

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of UL satisfied UE | Mean PSG of all UEs (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 193 | R1-2111046 | e-CDRX adapting to quasi (ideal)- period position | 16 | 8 | 4 |  | L | 4 | 8 | 99.60% | 32.97% |
| Source 18 | 198 | R1-2111046 | e-CDRX adapting to quasi (ideal)- period position | 16 | 8 | 4 |  | H | 8 | 8 | 90.67% | 31.72% |

            - InH

Observation

- In FR2, UL only evaluation, InH, AR UL 1 stream, it was observed from Source 18 that the enhanced CDRX scheme provides the mean power saving gain of 37.57% in the range of 36.79 ~ 38.35% with marginal loss in UL UE satisfied rate.

Table 106 Source specific data: eCDRX, FR2, UL-only, InH, AR UL 1 stream

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 180 | R1-2111046 | e-CDRX adapting to quasi (ideal)- period position | 16 | 8 | 4 |  | L | 4 | 8 | 100.00% | 38.35% |
| Source 18 | 185 | R1-2111046 | e-CDRX adapting to quasi (ideal)- period position | 16 | 8 | 4 |  | H | 8 | 8 | 92.36% | 36.79% |

#### Jitter Handling

This section provides the performance impact of potential jitter handling mechanisms.

XR DL traffic arrival has jitter which makes exact frame arrival timing random due to random delay contributed from frame encoders in Edge server, network transfer time in core network, etc. If traffic arrives too early, then packets should be delayed until UE wakes up from CDRX off state, which increases the latency for the packet transmission. This can potentially negatively affect the capacity given the tight PDB in DL. DL burst arrives later than the expected time of arrival (where DRX On duration start offset is configured), the UE should wait for DL burst arrival while performing unnecessary PDCCH monitoring. The unnecessary PDCCH monitoring increases UE power consumption. Jitter handling mechanisms address these issues. In this section, following potential jitter handling schemes have been captured.

- eCDRX with jitter handling

- Enhanced PDCCH monitoring adaption with jitter handling

- Enhanced CDRX with additional active time

- Fast/dense WUS for jitter handling with eCDRX

The general idea for jitter handling is to make DRX On duration start offset/PDCCH skipping end duration/new Active Time be aligned with actual DL traffic arrival time as much as possible by L1 indication or prediction based on traffic arrival statistics. This power saving gain captured in this section show the potential performance gain when jitter can be properly handled by such schemes.

##### DL+UL Evaluation

Table 107 Summary of PS schemes for jitter handlings, DL+UL evaluation

| Scen-arios | App | DL Bit rate (Mbps) | PS scheme | PSG(%), Note 1, 2 |  | Source |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Mean (%) | Range (%) |  |
| DU | VR | 30 | eCDRX with jitter handling | 30.50 | 28.12 ~ 32.88 | Source 18 |
|  |  |  | Enhanced PDCCH monitoring adaptation with jitter handling | 40.64 | 37.65 ~ 43.63 | Source 18 |
|  | AR | 30 | eCDRX with jitter handling | 23.36 | 20.65 ~ 27.46 | Source 18 |
|  |  |  | Enhanced PDCCH monitoring adaptation with jitter handling | 34.11 | 30.63 ~ 40.21 | Source 18 |
| InH | VR | 30 | eCDRX with jitter handling | 32.14 | 29.92 ~ 34.36 | Source 18 |
|  |  |  | Enhanced PDCCH monitoring adaptation with jitter handling | 40.74 | 39.86 ~ 41.62 | Source 18 |
|  |  |  | enhanced CDRX with additional active time | 20.50 | 20.50 | Source 20 |
|  |  | 45 | eCDRX with additional active time | 25.05 | 25.0 ~ 25.10 | Source 20 |
|  | AR | 30 | eCDRX with jitter handling | 24.30 | 21.43 ~ 30.41 | Source 18 |
|  |  |  | Enhanced PDCCH monitoring adaptation with jitter handling | 34.04 | 30.45 ~ 39.29 | Source 18 |
|  | CG | 30 | eCDRX with additional active time | 21.35 | 21.3 ~ 21.4% | Source 20 |
| Note 1 : PSG was computed for the cases only with marginal loss in % of UL satisfied UE. Note 2: The PSG is computed with respect to power consumption of AlwaysOn scheme. |  |  |  |  |  |  |

Observations

- In FR1, DL+UL evaluation, DU, VR30, it was observed from Source 18 that the eCDRX with jitter handling scheme provides the mean power saving gain of 30.50% in the range of 28.12 ~ 32.88% with marginal loss in DL+UL UE satisfied rate.

- In FR1, DL+UL evaluation, DU, VR30, it was observed from Source 18 that the enhanced PDCCH monitoring adaptation with jitter handling scheme provides the mean power saving gain of 40.64% in the range of 37.65 ~ 43.63% with marginal loss in DL+UL UE satisfied rate.

Table 108 Source specific data: FR1, DL+UL, DU, VR30

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | % of UL satisfied UE | % of DL + UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 241 | R1-2111046 | eCDRX with jitter handling | 16 | 3 | 3 |  | L | 7 | 13 |  |  | 100.00% | 32.88% |
| Source 18 | 243 | R1-2111046 | enhanced PDCCH monitoring adaptation with jitter handling | 0 | 0 | 0 |  | L | 7 | 13 |  |  | 100.00% | 43.63% |
| Source 18 | 249 | R1-2111046 | eCDRX with jitter handling | 16 | 3 | 3 |  | H | 13 | 13 |  |  | 91.82% | 28.12% |
| Source 18 | 251 | R1-2111046 | enhanced PDCCH monitoring adaptation with jitter handling | 0 | 0 | 0 |  | H | 13 | 13 |  |  | 91.94% | 37.65% |

Observations

- In FR1, DL+UL evaluation, DU, AR30, it was observed from Source 18 that the eCDRX with jitter handling scheme provides the mean power saving gain of 23.36% in the range of 20.65 ~ 27.46% with marginal loss in DL+UL UE satisfied rate.

- In FR1, DL+UL evaluation, DU, AR30, it was observed from Source 18 that the enhanced PDCCH monitoring with jitter handling scheme provides the mean power saving gain of 34.11% in the range of 30.63 ~ 40.21% with marginal loss in DL+UL UE satisfied rate.

Table 109 Source specific data: FR1, DL+UL, DU, AR30

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | % of UL satisfied UE | % of DL + UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 273 | R1-2111046 | Note 3 | 16 | 3 | 3 | Note 1 | L | 5 | 9 |  |  | 95.87% | 27.46% |
| Source 18 | 275 | R1-2111046 | Note 4 | 0 | 0 | 0 | Note 1 | L | 5 | 9 |  |  | 95.87% | 40.21% |
| Source 18 | 281 | R1-2111046 | Note 3 | 16 | 3 | 3 | Note 1 | H | 9 | 9 |  |  | 91.71% | 20.65% |
| Source 18 | 283 | R1-2111046 | Note 4 | 0 | 0 | 0 | Note 1 | H | 9 | 9 |  |  | 91.89% | 33.36% |
| Source 18 | 305 | R1-2111046 | Note 3 | 16 | 3 | 3 | Note 2 | L | 4 | 7 |  |  | 100.00% | 24.18% |
| Source 18 | 307 | R1-2111046 | Note 4 | 0 | 0 | 0 | Note 2 | L | 4 | 7 |  |  | 100.00% | 32.25% |
| Source 18 | 313 | R1-2111046 | Note 3 | 16 | 3 | 3 | Note 2 | H | 7 | 7 |  |  | 91.16% | 21.14% |
| Source 18 | 315 | R1-2111046 | Note 4 | 0 | 0 | 0 | Note 2 | H | 7 | 7 |  |  | 91.38% | 30.63% |
| Note 1 AR with single UL stream. Note 2 AR with two UL streams. Note 3. eCDRX with jitter handling Note 4. enhanced PDCCH monitoring adaptation with jitter handling |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Observations

- In FR1, DL+UL evaluation, InH, VR30, it was observed from Source 18 that the eCDRX with jitter handling scheme provides the mean power saving gain of 32.14% in the range of 29.92 ~ 34.36% with marginal loss in DL+UL UE satisfied rate.

- In FR1, DL+UL evaluation, InH, VR30, it was observed from Source 18 that the enhanced PDCCH monitoring adaptation with jitter handling scheme provides the mean power saving gain of 40.64% in the range of 37.65 ~ 43.63% with marginal loss in DL+UL UE satisfied rate.

- In FR1, DL+UL evaluation, InH, VR30, it was observed from Source 20 that the enhanced CDRX with additional active time scheme provides the mean power saving gain of 20.50% with marginal loss in DL+UL UE satisfied rate.

Table 110 Source specific data: FR1, DL+UL, InH, VR30

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | % of UL satisfied UE | % of DL + UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 225 | R1-2111046 | Note 1 | 16 | 3 | 3 |  | L | 5 | 10 |  |  | 100.00% | 34.36% |
| Source 18 | 227 | R1-2111046 | Note 2 | 0 | 0 | 0 |  | L | 5 | 10 |  |  | 100.00% | 41.62% |
| Source 18 | 233 | R1-2111046 | Note 1 | 16 | 3 | 3 |  | H | 10 | 10 |  |  | 91.27% | 29.92% |
| Source 18 | 235 | R1-2111046 | Note 2 | 0 | 0 | 0 |  | H | 10 | 10 |  |  | 91.11% | 39.86% |
| Source 20 | 9 | R1-2111351 | Note 3 | 16 | 6 | 4 |  | H | 11 | 11 | 91.67% | 100.00% | 91.67% | 20.50% |
| Source 20 | 10 | R1-2111351 | Note 3 | 16 | 6 | 4 |  | H | 11 | 11 | 91.67% | 100.00% | 91.67% | 20.50% |
| Note 1. eCDRX with jitter handling Note 2. enhanced PDCCH monitoring adaptation with jitter handling Note 3. eCDRX(change drx-startoffset per 100ms and additional active time) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Observation

- In FR1, DL+UL evaluation, InH, VR45, it was observed from Source 20 that the enhanced CDRX with additional active time scheme provides the mean power saving gain of 25.05% in the range of 25.0 ~ 25.10% with marginal loss in DL+UL UE satisfied rate.

Table 111 Source specific data: FR1, DL+UL, InH, VR45

| Source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | % of UL satisfied UE | % of DL + UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 20 | 17 | R1-2111351 | Note 1 | 16 | 6 | 4 |  | H | 7 | 7 | 86.30% | 100.00% | 86.30% | 25.10% |
| Source 20 | 18 | R1-2111351 | Note 1 | 16 | 6 | 4 |  | H | 7 | 7 | 86.30% | 100.00% | 86.30% | 25.00% |
| Note 1. eCDRX(change drx-startoffset per 100ms and additional active time) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Observations

- In FR1, DL+UL evaluation, InH, AR30, it was observed from Source 18 that the eCDRX with jitter handling scheme provides the mean power saving gain of 24.30% in the range of 21.43 ~ 30.41% with marginal loss in DL+UL UE satisfied rate.

- In FR1, DL+UL evaluation, InH, AR30, it was observed from Source 18 that the enhanced PDCCH monitoring adaptation with jitter handling scheme provides the mean power saving gain of 34.04% in the range of 30.45 ~ 39.29% with marginal loss in DL+UL UE satisfied rate.

Table 112 Source specific data: FR1, DL+UL, InH, AR30

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | % of UL satisfied UE | % of DL + UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 257 | R1-2111046 | Note 3 | 16 | 3 | 3 | Note 1 | L | 5 | 10 |  |  | 100.00% | 30.41% |
| Source 18 | 259 | R1-2111046 | Note 4 | 0 | 0 | 0 | Note 1 | L | 5 | 10 |  |  | 100.00% | 39.29% |
| Source 18 | 265 | R1-2111046 | Note 3 | 16 | 3 | 3 | Note 1 | H | 10 | 10 |  |  | 90.95% | 21.88% |
| Source 18 | 267 | R1-2111046 | Note 4 | 0 | 0 | 0 | Note 1 | H | 10 | 10 |  |  | 91.67% | 34.46% |
| Source 18 | 289 | R1-2111046 | Note 3 | 16 | 3 | 3 | Note 2 | L | 5 | 10 |  |  | 100.00% | 23.46% |
| Source 18 | 291 | R1-2111046 | Note 4 | 0 | 0 | 0 | Note 2 | L | 5 | 10 |  |  | 100.00% | 31.97% |
| Source 18 | 297 | R1-2111046 | Note 3 | 16 | 3 | 3 | Note 2 | H | 10 | 10 |  |  | 90.79% | 21.43% |
| Source 18 | 299 | R1-2111046 | Note 4 | 0 | 0 | 0 | Note 2 | H | 10 | 10 |  |  | 91.11% | 30.45% |
| Note 1. AR with single UL stream. Note 2. AR with two UL streams. Note 3. eCDRX with jitter handling Note 4. enhanced PDCCH monitoring adaptation with jitter handling |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Observation

- In FR1, DL+UL evaluation, InH, CG30, it was observed from Source 20 that the enhanced CDRX with additional active time scheme provides the mean power saving gain of 21.35% in the range of 21.3 ~ 21.4% with marginal loss in DL+UL UE satisfied rate.

Table 113 Source specific data: FR1, DL+UL, InH, CG30

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | % of UL satisfied UE | % of DL + UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 20 | 25 | R1-2111351 | Note 1 | 16 | 6 | 4 |  | H | 12 | 12 | 88.19% | 100.00% | 88.19% | 21.40% |
| Source 20 | 26 | R1-2111351 | Note 1 | 16 | 6 | 4 |  | H | 12 | 12 | 88.19% | 100.00% | 88.19% | 21.30% |
| Note 1. eCDRX(change drx-startoffset per 100ms and additional active time) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### DL-only Evaluation

###### FR1

Table 114 Summary of PS schemes for jitter handlings, FR1, DL-only

| FR | Scen-arios | App | DL Bit rate (Mbps) | PS scheme | PSG(%), Note 1,2 |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Mean (%) | Range (%) |  |
| FR1 | DU | VR | 30 | fast/dense WUS for jitter handling with eCDRX | 31 |  | Source 16 |
|  |  |  |  | eCDRX with jitter handling | 29.60 | 25.11~34.08 | Source 18 |
|  |  |  |  | Enhanced PDCCH monitoring adaptation with jitter handling | 42.61 | 37.83~47.38 | Source 18 |
|  |  |  | 45 | eCDRX with jitter handling | 32.27 | 29.30~35.23 | Source 18 |
|  |  |  |  | Enhanced PDCCH monitoring adaptation with jitter handling | 41.23 | 37.26~45.19 | Source 18 |
|  |  |  |  | Enhanced CDRX with additional active time | 29.9 | 29.9 | Source 20 |
|  | InH | VR | 30 | enhanced CDRX with additional active time | 29.8 | 29.8 | Source 20 |
|  |  |  |  | eCDRX with jitter handling | 32.21 | 29.06~35.35 | Source 18 |
|  |  |  |  | Enhanced PDCCH monitoring adaptation with jitter handling | 44.96 | 41.03~48.88 | Source 18 |
|  |  |  | 45 | eCDRX with jitter handling | 32.11 | 29.12 ~ 35.09 | Source 18 |
|  |  |  |  | Enhanced PDCCH monitoring adaptation with jitter handling | 42.04 | 38.76 ~ 45.32 | Source 18 |
|  |  |  |  | enhanced CDRX with additional active time | 29.7 | 29.7 | Source 20 |
|  |  | CG | 30 | eCDRX with additional active time | 32.4 | 32.4 | Source 20 |
|  | UMa | VR | 30 | eCDRX with jitter handling | 32.40 | 29.29~35.51 | Source 18 |
|  |  |  |  | Enhanced PDCCH monitoring adaptation with jitter handling | 43.86 | 40.59~47.13 | Source 18 |
|  |  |  | 45 | eCDRX with jitter handling | 30.97 | 29.51~32.43 | Source 18 |
|  |  |  |  | Enhanced PDCCH monitoring adaptation with jitter handling | 40.22 | 37.18~43.26 | Source 18 |
| Note 1 : PSG was computed for the cases only with marginal loss in % of UL satisfied UE. Note 2: The PSG is computed with respect to power consumption of AlwaysOn scheme. |  |  |  |  |  |  |  |

            - DU

Observations

- In FR1, DL evaluation, DU, VR30, it was observed from Source 16 that the fast/dense WUS for jitter handling + eCDRX scheme provides the mean power saving gain of 31.00% with marginal loss in DL UE satisfied rate.

- In FR1, DL evaluation, DU, VR30, it was observed from Source 18 that the eCDRX for jitter handling scheme provides the mean power saving gain of 29.60% in the range of 25.11~34.08% with marginal loss in DL UE satisfied rate.

- In FR1, DL evaluation, DU, VR30, it was observed from Source 18 that the enhanced PDCCH for jitter handling scheme provides the mean power saving gain of 42.61% in the range of 37.83~47.38% with marginal loss in DL UE satisfied rate.

Table 115 Source specific data:FR1, DL, DU, VR30

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 16 | 59 | R1-2112720 | Fast / dense WUS for jitter handling + eCDRX | 16/17/17 | 6 | 6 | 0 | H | 11 | 11 | 99.30% | 31.00% |
| Source 18 | 38 | R1-2111046 | eCDRX with jitter handling | 16 | 3 | 3 | 0 | L | 7 | 13 | 100.00% | 34.08% |
| Source 18 | 40 | R1-2111046 | enhanced PDCCH monitoring adaptation with jitter handling | 0 | 0 | 0 | 0 | L | 7 | 13 | 100.00% | 47.38% |
| Source 18 | 46 | R1-2111046 | eCDRX with jitter handling | 16 | 3 | 3 | 0 | H | 13 | 13 | 91.70% | 25.11% |
| Source 18 | 48 | R1-2111046 | enhanced PDCCH monitoring adaptation with jitter handling | 0 | 0 | 0 | 0 | H | 13 | 13 | 92.43% | 37.83% |

Observations

- In FR1, DL evaluation, DU, VR45, it was observed from Source 18 that the eCDRX for jitter handling scheme provides the mean power saving gain of 32.27% in the range of 29.30~35.23% with marginal loss in DL UE satisfied rate.

- In FR1, DL evaluation, DU, VR45, it was observed from Source 18 that the enhanced PDCCH for jitter handling scheme provides the mean power saving gain of 41.23% in the range of 37.26~45.19% with marginal loss in DL UE satisfied rate.

- In FR1, DL evaluation, DU, VR45, it was observed from Source 20 that the enhanced CDRX with additional active time scheme provides the mean power saving gain of 29.9% with marginal loss in DL UE satisfied rate.

Table 116 Source specific data:FR1, DL, DU, VR45

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 54 | R1-2111046 | Note 1 | 16 | 3 | 3 |  | L | 3 | 6 | 97.88% | 35.23% |
| Source 18 | 56 | R1-2111046 | Note 2 | - | - | - |  | L | 3 | 6 | 100.00% | 45.19% |
| Source 18 | 62 | R1-2111046 | Note 1 | 16 | 3 | 3 |  | H | 6 | 6 | 95.24% | 29.30% |
| Source 18 | 64 | R1-2111046 | Note 2 | - | - | - |  | H | 6 | 6 | 95.63% | 37.26% |
| Source 20 | 41 | R1-2111351 | Note 3 | 16 | 6 | 4 |  | H | 7 | 7 | 90% | 29.9% |
| Note 1. eCDRX with jitter handling Note 2. enhanced PDCCH monitoring adaptation with jitter handling Note 3. enhanced eCDRX(change drx startoffset per 100ms and additional active time) |  |  |  |  |  |  |  |  |  |  |  |  |

            - InH

Observations

- In FR1, DL evaluation, InH, VR30, it was observed from Source 18 that the eCDRX for jitter handling scheme provides the mean power saving gain of 32.21% in the range of 29.06~35.35% with marginal loss in DL UE satisfied rate.

- In FR1, DL evaluation, InH, VR30, it was observed from Source 18 that the enhanced PDCCH for jitter handling scheme provides the mean power saving gain of 44.96% in the range of 41.03~48.88% with marginal loss in DL UE satisfied rate.

- In FR1, DL evaluation, InH, VR30, it was observed from Source 20 that the enhanced CDRX with additional active time scheme provides the mean power saving gain of 29.8% with marginal loss in DL UE satisfied rate.

Table 117 Source specific data:FR1, DL, InH, VR30

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 6 | R1-2111046 | Note 1 | 16 | 3 | 3 |  | L | 5 | 10 | 100.00% | 35.35% |
| Source 18 | 8 | R1-2111046 | Note 2 | - | - | - |  | L | 5 | 10 | 100.00% | 48.88% |
| Source 18 | 14 | R1-2111046 | Note 1 | 16 | 3 | 3 |  | H | 10 | 10 | 91.67% | 29.06% |
| Source 18 | 16 | R1-2111046 | Note 2 | - | - | - |  | H | 10 | 10 | 92.50% | 41.03% |
| Source 20 | 31 | R1-2111351 | Note 3 | 16 | 6 | 4 |  | H | 11 | 11 | 91.67% | 29.8% |
| Note 1. eCDRX with jitter handling Note 2. enhanced PDCCH monitoring adaptation with jitter handling Note 3. enhanced eCDRX(change drx startoffset per 100ms and additional active time) |  |  |  |  |  |  |  |  |  |  |  |  |

Observations

- In FR1, DL only evaluation, InH, VR45, it was observed from Source 18 that the eCDRX for jitter handling provides the mean power saving gain of 32.11% in the range of 29.12 ~ 35.09% with marginal loss in DL UE satisfied rate.

- In FR1, DL only evaluation, InH, VR45, it was observed from Source 18 that the enhanced PDCCH monitoring adaptation with jitter handling provides the mean power saving gain of 42.04% in the range of 38.76 ~ 45.32% with marginal loss in DL UE satisfied rate.

- In FR1, DL only evaluation, InH, VR45, it was observed from Source 20 that the enhanced CDRX with additional active time scheme provides the mean power saving gain of 29.7% with marginal loss in DL UE satisfied rate.

Table 118 Source specific data: FR1, DL-only, InH, VR45

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 22 | R1-2111046 | Note 1 | 16 | 3 | 3 |  | L | 3 | 5 | 100.00% | 35.09% |
| Source 18 | 24 | R1-2111046 | Note 2 | 0 | 0 | 0 |  | L | 3 | 5 | 100.00% | 45.32% |
| Source 18 | 30 | R1-2111046 | Note 1 | 16 | 3 | 3 |  | H | 5 | 5 | 94.44% | 29.12% |
| Source 18 | 32 | R1-2111046 | Note 2 | 0 | 0 | 0 |  | H | 5 | 5 | 96.67% | 38.76% |
| Source 20 | 35 | R1-2111351 | Note 3 | 16 | 6 | 4 |  | H | 7 | 7 | 86.3% | 29.7% |
| Note 1. eCDRX with jitter handling Note 2. enhanced PDCCH monitoring adaptation with jitter handling Note 3. enhanced eCDRX(change drx startoffset per 100ms and additional active time) |  |  |  |  |  |  |  |  |  |  |  |  |

Observation

- In FR1, DL only evaluation, InH, CG30, it was observed from Source 20 that the enhanced CDRX with additional active time scheme provides the mean power saving gain of 32.4% with marginal loss in DL UE satisfied rate.

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 20 | 39 | R1-2111351 | enhanced eCDRX(change drx startoffset per 100ms and additional active time) | 16 | 6 | 4 |  | H | 12 | 12 | 88.19% | 32.4% |

            - UMa

Observations

- In FR1, DL evaluation, UMa, VR30, it was observed from Source 18 that the eCDRX for jitter handling scheme provides the mean power saving gain of 32.40% in the range of 29.29~35.51% with marginal loss in DL UE satisfied rate.

- In FR1, DL evaluation, UMa, VR30, it was observed from Source 18 that the enhanced PDCCH for jitter handling scheme provides the mean power saving gain of 43.86% in the range of 40.59~47.13% with marginal loss in DL UE satisfied rate.

Table 119 Source specific data:FR1, DL, UMa, VR30

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 70 | R1-2111046 | eCDRX with jitter handling | 16 | 3 | 3 |  | L | 4 | 8 | 98.02% | 35.51% |
| Source 18 | 72 | R1-2111046 | enhanced PDCCH monitoring adaptation with jitter handling | - | - | - |  | L | 4 | 8 | 98.81% | 47.13% |
| Source 18 | 78 | R1-2111046 | eCDRX with jitter handling | 16 | 3 | 3 |  | H | 8 | 8 | 93.25% | 29.29% |
| Source 18 | 80 | R1-2111046 | enhanced PDCCH monitoring adaptation with jitter handling | - | - | - |  | H | 8 | 8 | 93.75% | 40.59% |

Observations

- In FR1, DL evaluation, UMa, VR45, it was observed from Source 18 that the eCDRX for jitter handling scheme provides the mean power saving gain of 30.97% in the range of 29.51~32.43% with marginal loss in DL UE satisfied rate.

- In FR1, DL evaluation, UMa, VR45, it was observed from Source 18 that the enhanced PDCCH for jitter handling scheme provides the mean power saving gain of 40.22% in the range of 37.18~43.26% with marginal loss in DL UE satisfied rate.

Table 120 Source specific data:FR1, DL, UMa, VR45

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 86 | R1-2111046 | Note 1 | 16 | 3 | 3 |  | L | 2 | 4 | 96.83% | 32.43% |
| Source 18 | 88 | R1-2111046 | Note 2 | - | - | - |  | L | 2 | 4 | 96.83% | 43.26% |
| Source 18 | 94 | R1-2111046 | Note 1 | 16 | 3 | 3 |  | H | 4 | 4 | 92.46% | 29.51% |
| Source 18 | 96 | R1-2111046 | Note 2 | - | - | - |  | H | 4 | 4 | 94.05% | 37.18% |
| Note 1. eCDRX with jitter handling Note 2. enhanced PDCCH monitoring adaptation with jitter handling |  |  |  |  |  |  |  |  |  |  |  |  |

###### FR2

            - DU

Table 121 Summary of PS schemes for jitter handlings, FR2, DL-only

| FR | Scen-arios | App | DL Bit rate (Mbps) | PS scheme | PS Gain (%), Note 1,2 |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Mean (%) | Range (%) |  |
| FR2 | DU | VR | 30 | Enhanced PDCCH monitoring adaptation with jitter handling | 57.58 | 55.51~59.65 | Source 18 |
|  |  |  | 45 | Enhanced PDCCH monitoring adaptation with jitter handling | 52.03 | 50.46~53.59 | Source 18 |
|  | InH | VR | 30 | Enhanced PDCCH monitoring adaptation with jitter handling | 59.69 | 57.53~61.85 | Source 18 |
|  |  |  | 45 | Enhanced PDCCH monitoring adaptation with jitter handling | 53.32 | 52.14~54.50 | Source 18 |
| Note 1 : PSG was computed for the cases only with marginal loss in % of UL satisfied UE. Note 2: The PSG is computed with respect to power consumption of AlwaysOn scheme. |  |  |  |  |  |  |  |

Observation

- In FR2, DL evaluation, DU, VR30, it was observed from Source 18 that the enhanced PDCCH for jitter handling scheme provides the mean power saving gain of 57.58% in the range of 55.51~59.65% with marginal loss in DL UE satisfied rate.

Table 122 Source specific data:FR2, DL, DU, VR30

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 126 | R1-2111046 | enhanced PDCCH monitoring adaptation with jitter handling | - | - | - |  | L | 7 | 13 | 99.55% | 59.65% |
| Source 18 | 132 | R1-2111046 | eCDRX with jitter handling | 16 | 3 | 3 |  | H | 13 | 13 | 95.24% | 55.51% |

Observation

- In FR2, DL evaluation, DU, VR45, it was observed from Source 18 that the enhanced PDCCH for jitter handling scheme provides the mean power saving gain of 52.03% in the range of 50.46~53.59% with marginal loss in DL UE satisfied rate.

Table 123 Source specific data:FR2, DL, DU, VR45

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 138 | R1-2111046 | enhanced PDCCH monitoring adaptation with jitter handling | - | - | - |  | L | 4 | 8 | 100.00% | 53.59% |
| Source 18 | 144 | R1-2111046 | eCDRX with jitter handling | 16 | 3 | 3 |  | H | 8 | 8 | 93.25% | 50.46% |

            - InH

Observation

- In FR2, DL evaluation, InH, VR30, it was observed from Source 18 that the enhanced PDCCH for jitter handling scheme provides the mean power saving gain of 59.69% in the range of 57.53~61.85% with marginal loss in DL UE satisfied rate.

Table 124 Source specific data:FR2, DL, InH, VR30

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 102 | R1-2111046 | enhanced PDCCH monitoring adaptation with jitter handling | - | - | - |  | L | 4 | 8 | 100.00% | 61.85% |
| Source 18 | 108 | R1-2111046 | eCDRX with jitter handling | 16 | 3 | 3 |  | H | 8 | 8 | 92.01% | 57.53% |

Observation

- In FR2, DL evaluation, InH, VR45, it was observed from Source 18 that the enhanced PDCCH for jitter handling scheme provides the mean power saving gain of 53.32% in the range of 52.14~54.50% with marginal loss in DL UE satisfied rate.

Table 125 Source specific data:FR2, DL, InH, VR45

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | 114 | R1-2111046 | enhanced PDCCH monitoring adaptation with jitter handling | - | - | - |  | L | 2 | 4 | 100.00% | 54.50% |
| Source 18 | 120 | R1-2111046 | eCDRX with jitter handling | 16 | 3 | 3 |  | H | 4 | 4 | 94.44% | 52.14% |

#### XR dedicated PDCCH Monitoring Window

In this section, we capture the evaluation results for dynamic scheduling of  XR specific dedicated PDCCH monitoring window scheme with PDCCH skipping and go-to-sleep. In this scheme, XR dedicated PDCCH monitoring window/cycle is defined, which is disassociated with the DRX configuration, but aligned with XR traffic pattern. Dynamic scheduling with XR specific dedicated PDCCH monitoring window scheme would have UE monitor PDCCH in the given window in both within Active time and outside Active Time when DRX is configured.

Table 126 Summary of source specific data for XR dedicated PDCCH Monitoring Window, FR1, InH, VR

| Scen-arios | App | DL Bit rate (Mbps) | Direction | Assumptions | PSG(%), Note 1,2 |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Mean (%) | Range (%) |  |
| InH | VR | 30 | DL | PDCCH monitoring window with PDCCH skipping and go-to-sleep | 26.73 | 24.01~29.44 | Source 3 |
| Note 1 : PSG was computed for the cases only with marginal loss in % of DL satisfied UE. Note 2: The PSG is computed with respect to power consumption of AlwaysOn scheme. |  |  |  |  |  |  |  |

Observation

- In FR1, DL evaluation, DU, VR30, it was observed from Source 3 that the XR dedicated PDCCH monitoring window scheme provides the mean power saving gain of 26.73% in the range of 24.01~29.44% with marginal loss in DL UE satisfied rate.

Table 127 Source specific data: FR1, DL, InH, VR30, XR dedicated PDCCH monitoring window

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 3 | 4 | R1-2111234 | XR-dedicated PDCCH monitoring window with go-to-sleep | 0 | 0 | 0 | Monitoring cycle =16.67ms; Monitoring window=16.67ms; | H | 12 | 12 | 90.00% | 24.01% |
| Source 3 | 5 | R1-2111234 | XR-dedicated PDCCH monitoring window with PDCCH skipping and go-to-sleep | 0 | 0 | 0 | Monitoring cycle =16.67ms; Monitoring window=16.67ms; | H | 12 | 12 | 89.16% | 29.44% |

#### Additional Packet Delay Budget with Playout Buffer

This section captures the evaluation results of the impact of additional PDB (APDB) on UE power consumption. The XR application layer at UE would have the XR packet playout buffer to battle the delay jitter and out-of sequence XR packet arrival.  The playout buffer at UE would ensure the in-sequence and time interval alignment of XR video frames when it plays out to the user.   The proposed scheme is for UE to feedback not only the XR-application type (XR-application awareness) but also the implemented playout buffer at application layer to the gNB. If the size of playout buffer is fed back from UE and known at gNB, then, additional PDB could be used for packet scheduling, which could give gNB more time to schedule UE within the delay budget requirements of the XR service and more likely to successfully transmit packets with link adaptation gain. The  addition of the power saving techniques, such as PDCCH skipping and go-to-sleep, could help UE in achieving power saving by reducing PDCCH monitoring during extended scheduling period by additional PDB.

Observation

- In FR1, DL evaluation, DU, VR30, it was observed from Source 3 that additional packet delay budget with play out buffer provides the mean power saving gain of 30.42% in the range of 26.43~34.56% with marginal loss in DL UE satisfied rate.

Table 128 Source specific data for additional packet delay budget with play out buffer: FR1, DU, DL, VR30

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Additional Assumptions | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%), Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 3 | 6 | R1-2111234 | CDRX(16,8,4) with go-to-sleep with UE playout buffer | 0 | 0 | 0 |  | H | 12 | 12 | 94.17% | 26.43% |
| Source 3 | 7 | R1-2111234 | C-DRX(16,8,4) with PDCCH skipping and go-to-sleep with UE playout buffer | 0 | 0 | 0 |  | H | 12 | 12 | 93.30% | 34.56% |
| Source 3 | 8 | R1-2111234 | PDCCH skipping with UE playout buffer | 0 | 0 | 0 |  | H | 12 | 12 | 91.67% | 30.26% |
| Note 1, PSG is computed w.r.t scheme is Always-on scheme and gNB does not have the additional PDB from UE feedback on the Playout buffer |  |  |  |  |  |  |  |  |  |  |  |  |

#### Traffic Arrival Offset Staggering

This section captures the evaluation results of the impact of different traffic arrival offsets across different UEs. The XR DL traffic arrival offsets potentially determines the time UE wakes up and how long the UE need to be awake. When the frame arrivals are aligned with each other, the XR traffic of the UEs contend for resources, decreasing the capacity and increasing power consumption due to the increased UE awake time. If the frame arrivals of UEs are staggered relative to each other, the cell can serve UEs minimizing the overlap of scheduling durations of each UE, which consequently reduce UEs’ awake time and make UEs stay longer in sleep state.

Table 129 Summary of source specific data for traffic arrival offset staggering, FR1, DU, VR, DL

| Scen-arios | App | DL Bit rate (Mbps) | Fps | Traffic arrival offset | PSG(%), Note 1,2 |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Mean (%) | Range (%) |  |
| DU | VR | 30 | 60 | All Sync | 0% |  | Source 16 |
|  |  |  |  | Random | 5.96% | 2.64~9.27% | Source 16 |
|  |  |  |  | Evenly spaced | 6.37% | 2.80~9.94% | Source 16 |
|  |  |  | 30 | All Sync | 0% |  | Source 16 |
|  |  |  |  | Random | 8.48% | 2.74~14.21% | Source 16 |
|  |  |  |  | Evenly spaced | 9.18% | 3.03~15.33% | Source 16 |
|  |  | 45 | 60 | All Sync | 0% |  | Source 16 |
|  |  |  |  | Random | 5.41% | 2.69~8.12% | Source 16 |
|  |  |  |  | Evenly spaced | 5.7% | 2.85~8.55% | Source 16 |
|  |  |  | 30 | All Sync | 0% |  | Source 16 |
|  |  |  |  | Random | 6.49% | 2.37~10.61% | Source 16 |
|  |  |  |  | Evenly spaced | 8.46% | 3.09~13.82% | Source 16 |
| Note 1 : PSG was computed for the cases only with marginal loss in % of DL satisfied UE. Note 2: The PSG is computed with respect to power consumption of All Sync scheme (AlwaysOn scheme with all UEs have the same traffic arrival offset). |  |  |  |  |  |  |  |

Observations

- In FR1, DL evaluation, DU, VR30Mbps-60Fps, it was identified from Source 16 that making random traffic arrival offset provide the mean power saving gain of 5.96% in the range of 2.64~9.27% with respect to all synced traffic arrival offset with marginal loss in DL UE satisfied rate.

- In FR1, DL evaluation, DU, VR30Mbps-60Fps, it was identified from Source 16 that making evenly spaced traffic arrival offset provide the mean power saving gain of 6.37% in the range of 2.80~9.94% with respect to all synced traffic arrival offset with marginal loss in DL UE satisfied rate.

Table 130 Source specific data: FR1, DU, DL, VR30Mbps-60Fps

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Traffic arrival offset | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 16 | 93 | R1-2112720 | Always ON | 0 | 0 | 0 | All Sync | H | 7 | 7 | 90.00% | 0.00% |
| Source 16 | 94 | R1-2112720 | Always ON | 0 | 0 | 0 | Random | H | 7 | 8 | 100.00% | 2.64% |
| Source 16 | 95 | R1-2112720 | Always ON | 0 | 0 | 0 | Evenly Spaced | L | 7 | 9 | 100.00% | 2.80% |
| Source 16 | 105 | R1-2112720 | Note 1 | 0 | 0 | 0 | All Sync | H | 7 | 7 | 90.00% | 0.00% |
| Source 16 | 106 | R1-2112720 | Note 1 | 0 | 0 | 0 | Random | H | 7 | 8 | 100.00% | 9.27% |
| Source 16 | 107 | R1-2112720 | Note 1 | 0 | 0 | 0 | Evenly Spaced | L | 7 | 9 | 100.00% | 9.94% |
| Note 1. Genie (CDRX with ideal PDCCH Skipping) |  |  |  |  |  |  |  |  |  |  |  |  |

Observations

- In FR1, DL evaluation, DU, VR30Mbps-30Fps, it was identified from Source 16 that making evenly spaced traffic arrival offset provide the mean power saving gain of 8.48% in the range of 2.74~14.21% with respect to all synced traffic arrival offset with marginal loss in DL UE satisfied rate.

- In FR1, DL evaluation, DU, VR30Mbps-30Fps, it was identified from Source 16 that making evenly spaced traffic arrival offset provide the mean power saving gain of 9.18% in the range of 3.03~15.33% with respect to all synced traffic arrival offset with marginal loss in DL UE satisfied rate.

Table 131 Source specific data: FR1, DU, DL, VR30Mbps-30Fps

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Traffic arrival offset | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 16 | 99 | R1-2112720 | Always ON | 0 | 0 | 0 | All Sync | H | 3 | 3 | 92.00% | 0% |
| Source 16 | 100 | R1-2112720 | Always ON | 0 | 0 | 0 | Random | L | 3 | 6 | 98.00% | 2.74% |
| Source 16 | 101 | R1-2112720 | Always ON | 0 | 0 | 0 | Evenly Spaced | L | 3 | 8 | 98.00% | 3.03% |
| Source 16 | 111 | R1-2112720 | Note 1 | 0 | 0 | 0 | All Sync | H | 3 | 3 | 92.00% | 0% |
| Source 16 | 112 | R1-2112720 | Note 1 | 0 | 0 | 0 | Random | L | 3 | 6 | 98.00% | 14.21% |
| Source 16 | 113 | R1-2112720 | Note 1 | 0 | 0 | 0 | Evenly Spaced | L | 3 | 8 | 98.00% | 15.33% |
| Note 1. Genie (CDRX with ideal PDCCH Skipping) |  |  |  |  |  |  |  |  |  |  |  |  |

Observations

- In FR1, DL evaluation, DU, VR45Mbps-60Fps, it was identified from Source 16 that making evenly spaced traffic arrival offset provide the mean power saving gain of 5.41% in the range of 2.69~8.12% with respect to all synced traffic arrival offset with marginal loss in DL UE satisfied rate.

- In FR1, DL evaluation, DU, VR45Mbps-60Fps, it was identified from Source 16 that making evenly spaced traffic arrival offset provide the mean power saving gain of 5.7% in the range of 2.85~8.55% with respect to all synced traffic arrival offset with marginal loss in DL UE satisfied rate.

Table 132 Source specific data: FR1, DL, VR45Mbps-60Fps

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Traffic arrival offset | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 16 | 90 | R1-2112720 | Always ON | 0 | 0 | 0 | All Sync | H | 4 | 4 | 98.00% | 0% |
| Source 16 | 91 | R1-2112720 | Always ON | 0 | 0 | 0 | Random | H | 4 | 5 | 99.00% | 2.69% |
| Source 16 | 92 | R1-2112720 | Always ON | 0 | 0 | 0 | Evenly Spaced | L | 4 | 6 | 100.00% | 2.85% |
| Source 16 | 102 | R1-2112720 | Note 1 | 0 | 0 | 0 | All Sync | H | 4 | 4 | 98.00% | 0% |
| Source 16 | 103 | R1-2112720 | Note 1 | 0 | 0 | 0 | Random | H | 4 | 5 | 99.00% | 8.12% |
| Source 16 | 104 | R1-2112720 | Note 1 | 0 | 0 | 0 | Evenly Spaced | L | 4 | 6 | 100.00% | 8.55% |
| Note 1. Genie (CDRX with ideal PDCCH Skipping) |  |  |  |  |  |  |  |  |  |  |  |  |

Observations

- In FR1, DL evaluation, DU, VR45Mbps-30Fps, it was identified from Source 16 that making evenly spaced traffic arrival offset provide the mean power saving gain of 6.49% in the range of 2.37~10.61% with respect to all synced traffic arrival offset with marginal loss in DL UE satisfied rate.

- In FR1, DL evaluation, DU, VR45Mbps-30Fps, it was identified from Source 16 that making evenly spaced traffic arrival offset provide the mean power saving gain of 8.46% in the range of 3.09~13.82% with respect to all synced traffic arrival offset with marginal loss in DL UE satisfied rate.

Table 133 Source specific data: FR1, DU, DL, VR45Mbps-30Fps

| source | data row index | Tdoc source | Power saving scheme | CDRX cycle (ms) | ODT (ms) | IAT (ms) | Traffic arrival offset | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 16 | 96 | R1-2112720 | Always ON | 0 | 0 | 0 | All Sync | H | 1 | 1 | 97.00% | 0% |
| Source 16 | 97 | R1-2112720 | Always ON | 0 | 0 | 0 | Random | L | 1 | 3 | 98.00% | 2.37% |
| Source 16 | 98 | R1-2112720 | Always ON | 0 | 0 | 0 | Evenly Spaced | L | 1 | 5 | 99.00% | 3.09% |
| Source 16 | 108 | R1-2112720 | Note 1 | 0 | 0 | 0 | All Sync | H | 1 | 1 | 97.00% | 0% |
| Source 16 | 109 | R1-2112720 | Note 1 | 0 | 0 | 0 | Random | L | 1 | 3 | 98.00% | 10.61% |
| Source 16 | 110 | R1-2112720 | Note 1 | 0 | 0 | 0 | Evenly Spaced | L | 1 | 5 | 99.00% | 13.82% |
| Note 1. Genie (CDRX with ideal PDCCH Skipping) |  |  |  |  |  |  |  |  |  |  |  |  |

#### SR group switching

This section captures the evaluation results of the SR group switching and baseline. The XR UL traffic arrives frequently, especially for pose/control. For dynamic scheduling, UE transmits a SR if UE has data to be transmitted. UE will monitor PDCCH for potential UL grant after the transmission of SR. Frequent SR transmission will increase both UL and DL power consumption. Switching between a dense SR periodicity and a sparse SR periodicity can achieve a tradeoff between latency of UL data transmission and UE power consumption.

Observation

- In FR1, UL evaluation, InH, ULPose with 250FPS, it was observed from Source 20 that SR group switching provide the mean power saving gain of 12.1% with respect to UL_baseline (UE can perform UL transmission at every UL slot/symbol if needed) with marginal loss in UL UE satisfied rate.

Table 134 Source specific data: FR1, InH, UL, UL Pose 250FPS

| source | data row index | Tdoc source | Power saving scheme | SR switch cycle (ms) | SR group #1 periodicity (slots) | SR group #1 duration(ms) | SR group #2 periodicity (slot) | SR group #2 duration(ms) | Load H/L | N1 | C1 | % of UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 20 | 42 | R1-2111351 | UL_baseline, Note 1 |  |  |  |  |  | L | 11 | >40 | 100% | 0% |
| Source 20 | 43 | R1-2111351 | SR group switching, Note 1 | 16 | 1 | 6 | 15 | 10 | L | 11 | >40 | 100% | 12.1% |
| Note 1: If UE transmits SR, UE will monitor PDCCH for the subsequent 2.5ms, otherwise, UE does not monitor PDCCH. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Observation

- In FR1, UL evaluation, DU, UL Pose 250FPS it was observed from Source 20 that SR group switching provide the mean power saving gain of 11.37% with respect to UL baseline(UE can perform UL transmission at every UL slot/symbol if needed) with marginal loss in UL UE satisfied rate.

Table 135 Source specific data: FR1, DU, UL, UL Pose 250FPS

| source | data row index | Tdoc source | Power saving scheme | SR switch cycle (ms) | SR group #1 periodicity (slots) | SR group #1 duration(ms) | SR group #2 periodicity (slot) | SR group #2 duration(ms) | Load H/L | N1 | C1 | % of UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 20 | 44 | R1-2111351 | UL_baseline, Note 1 |  |  |  |  |  | L | 11 | >40 | 100% | 0% |
| Source 20 | 45 | R1-2111351 | SR group switching, Note 1 | 16 | 1 | 6 | 15 | 10 | L | 11 | >40 | 100% | 11.37% |
| Note 1: If UE transmits SR, UE will monitor PDCCH for the subsequent 2.5ms, otherwise, UE does not monitor PDCCH. |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### UL active time

This section captures the evaluation results of the UL active time and baseline. Various signals/data needs to be transmitted in UL. UE needs to wake up frequently to transmit signals/data which requires more power for warm-up and ramp-down. The independent transmission occasion of each signals/data shall also split sleep time of the UE. If the transmission of UL signals/data can be confined within an UL active time, UE is more likely to go to a deeper sleep. UL active time is defined by a periodic cycle and a duration, where UE only transmits UL signals/data during the duration time of the cycle, and UE doesn’t transmit UL signals/data outside the duration of the cycle. In this section, UL active time with configurations of (cycle, duration) = (8ms, 4ms), (8ms, 3ms) is evaluated.

Observation

- In FR1, UL evaluation, InH, UL Pose 250FPS, it was observed from Source 20 that UL active time provide the mean power saving gain of 16.335% in the range of 13.67 ~19% with respect to UL_baseline(UE can perform UL transmission at every UL slot/symbol if needed) with marginal loss in UL UE satisfied rate.

Table 136 Source specific data: FR1, InH, UL, UL Pose250FPS

| source | data row index | Tdoc source | Power saving scheme | UL active time cycle (ms) | UL active time duration (slots) | Load H/L | N1 | C1 | % of UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 20 | 46 | R1-2111351 | UL_baseline, Note 1 |  |  | L | 11 |  | 100% | 0% |
| Source 20 | 47 | R1-2111351 | UL active time, Note 1 | 8 | 4 | L | 11 |  | 100% | 13.67% |
| Source 20 | 50 | R1-2111351 | UL_baseline, Note 1 |  |  | L | 3 |  | 100% | 0% |
| Source 20 | 51 | R1-2111351 | UL active time, Note 1 | 8 | 3 | L | 3 |  | 100% | 19% |
| Note 1: configured grant(periodicity = 2.5ms), UE does not need to monitor PDCCH. |  |  |  |  |  |  |  |  |  |  |

Observation

- In FR1, UL evaluation, DU, UL Pose 250FPS, it was observed from Source 20 that UL active time provide the mean power saving gain of 14% with respect to UL baseline(UE can perform UL transmission at every UL slot/symbol if needed) with marginal loss in UL UE satisfied rate.

Table 137 Source specific data: FR1, DU, UL, UL Pose250FPS

| source | data row index | Tdoc source | Power saving scheme | UL active time cycle (ms) | UL active time duration (slots) | Load H/L | N1 | C1 | % of UL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 20 | 48 | R1-2111351 | UL_baseline, Note 1 |  |  | L | 11 |  | 100% | 0% |
| Source 20 | 49 | R1-2111351 | UL active time, Note 1 | 8 | 4 | L | 11 |  | 100% | 14% |
| Note 1: configured grant (periodicity = 2.5ms), UE does not need to monitor PDCCH. |  |  |  |  |  |  |  |  |  |  |

#### Enhanced PDCCH monitoring

This section captures the evaluation results of enhanced PDCCH monitoring, where it configures a MonitoringSlotPeriodicity pattern with different MonitoringSlotPeriodicity values instead of a single MonitoringSlotPeriodicity value. For example, MonitoringSlotPeriodicity pattern is set as {17, 17, 16}ms.

Observation

- In FR1, for DL VR/AR@30Mbps and DL CG 30Mbps in DU, it was observed from Source 9 that enhanced PDCCH monitoring provides the power saving gain in the range of 5%~22%

Table 138 Source specific data: FR1, Dense Urban, DL, VR/AR30

| source | Tdoc source | Power saving scheme | MonitoringSlotPeriodicity pattern | Duration (ms) | MonitoringSlotOffset (ms) | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 9 | R1-2110811 | e-PDCCH monitoring | 17/17/16 ms | 8 | -2 | L | 3 | 5 | 75.24% | 22.05% |
| Source 9 | R1-2110811 | e-PDCCH monitoring | 17/17/16 ms | 10 | -4 | L | 3 | 5 | 74.92% | 15.38% |
| Source 9 | R1-2110811 | e-PDCCH monitoring | 17/17/16 ms | 12 | -4 | L | 3 | 5 | 94.76% | 9.09% |
| Source 9 | R1-2110811 | e-PDCCH monitoring | 17/17/16 ms | 14 | -4 | L | 3 | 5 | 97.94% | 5.18% |
| Source 9 | R1-2110811 | e-PDCCH monitoring | 17/17/16 ms | 8 | -2 | H | 5 | 5 | 59.05% | 21.84% |
| Source 9 | R1-2110811 | e-PDCCH monitoring | 17/17/16 ms | 10 | -4 | H | 5 | 5 | 59.90% | 15.25% |
| Source 9 | R1-2110811 | e-PDCCH monitoring | 17/17/16 ms | 12 | -4 | H | 5 | 5 | 84.57% | 8.96% |
| Source 9 | R1-2110811 | e-PDCCH monitoring | 17/17/16 ms | 14 | -4 | H | 5 | 5 | 90.67% | 5.08% |

Table 139 Source specific data: FR1, Dense Urban, DL, CG30

| source | Tdoc source | Power saving scheme | MonitoringSlotPeriodicity pattern | Duration (ms) | MonitoringSlotOffset (ms) | Load H/L | N1 | C1 | % of DL satisfied UE | Mean PSG of all Ues (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 9 | R1-2110811 | e-PDCCH monitoring | 17/17/16 ms | 8 | -2 | L | 3 | 7 | 92.22% | 21.91% |
| Source 9 | R1-2110811 | e-PDCCH monitoring | 17/17/16 ms | 10 | -4 | L | 3 | 7 | 97.62% | 15.22% |
| Source 9 | R1-2110811 | e-PDCCH monitoring | 17/17/16 ms | 12 | -4 | L | 3 | 7 | 99.37% | 9.05% |
| Source 9 | R1-2110811 | e-PDCCH monitoring | 17/17/16 ms | 14 | -4 | L | 3 | 7 | 99.84% | 5.16% |
| Source 9 | R1-2110811 | e-PDCCH monitoring | 17/17/16 ms | 8 | -2 | H | 7 | 7 | 60.88% | 21.38% |
| Source 9 | R1-2110811 | e-PDCCH monitoring | 17/17/16 ms | 10 | -4 | H | 7 | 7 | 71.84% | 14.58% |
| Source 9 | R1-2110811 | e-PDCCH monitoring | 17/17/16 ms | 12 | -4 | H | 7 | 7 | 83.67% | 8.73% |
| Source 9 | R1-2110811 | e-PDCCH monitoring | 17/17/16 ms | 14 | -4 | H | 7 | 7 | 88.44% | 4.95% |

# XR Coverage Evaluation

## Purpose of Study

The coverage study is for understanding the DL and UL coverage performance of XR applications. Note that the coverage depends on the evaluation assumptions/setup such as considered link direction (DL vs UL), bit rate, PDB, PER requirement, gNB/UE tx power, etc. Thus, the metric should be understood as a conditional metric for the given assumption. Through this study, we can identify the coverage of XR applications in terms of coupling gain and bottleneck direction of the considered applications.

## KPI

The KPI of the coverage evaluation is XR coverage which is defined as the 5% point of CDF of coupling gains for the satisfied Ues.

In this study, we consider two slightly different evaluation methodologies. The details of the two coverage evaluation methodologies are found in Annex A.3.

## XR Coverage Evaluation

### Coverage based on Methodology 1

#### FR1

##### DU

Following table captures the company reported XR coverage evaluation results for FR1, DU.

Table 140 XR coverage results for Method 1, FR1, DU, DDDSU

| Link direction | Applications | PDB (ms) | #UEs/cell | XR Coverage |  | Source | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Mean (dB) | Value (dB) |  |  |
| DL | VR/AR30 | 10 | 9 (Capacity) | -121.38 | -121.9 | Source 18 | Note 1 |
|  |  |  | 12 (Capacity) |  | -118.08 | Source 20 |  |
|  |  |  | 8 (Capacity) |  | -126 | Source 15 |  |
|  |  |  | 7 (Capacity) |  | -119.55 | Source 19 |  |
|  |  |  | 1 | -120.80 | -122.9 | Source 18 |  |
|  |  |  |  |  | -123 | Source 15 |  |
|  |  |  |  |  | -119 | Source 7 |  |
|  |  |  |  |  | -120.85 | Source 19 |  |
|  |  |  |  |  | -118.31 | Source 3 |  |
|  | VR/AR45 | 10 | 7 (Capacity) | -121.86 | -120.39 | Source 20 |  |
|  |  |  | 6 (Capacity) |  | -126 | Source 15 |  |
|  |  |  | 5 (Capacity) |  | -119.2 | Source 19 |  |
|  |  |  | 1 | -121.93 | -123 | Source 15 |  |
|  |  |  |  |  | -120.85 | Source 19 |  |
|  | CG30 | 15 | 14 (Capacity) | -122.09 | -120.41 | Source 20 |  |
|  |  |  | 8 (Capacity) |  | -126 | Source 15 |  |
|  |  |  | 8 (Capacity) |  | -119.86 | Source 19 |  |
|  |  |  | 1 | -121.95 | -123 | Source 15 |  |
|  |  |  |  |  | -119 | Source 7 |  |
|  |  |  |  |  | -120.85 | Source 19 |  |
| UL | Pose | 10 | 25 | -119.72 | -119.72 | Source 20 |  |
|  |  |  | 1 | -119 | -119 | Source 7 | Note 2 |
|  | AR 1 stream | 30 | 9 (Capacity) | -118.53 | -117 | Source 18 | Note 1 |
|  |  |  | 10 (Capacity) |  | -120.07 | Source 20 |  |
|  |  |  | 1 | -116.56 | -122.9 | Source 18 |  |
|  |  |  |  |  | -118 | Source 7 | Note 3 |
|  |  |  |  |  | -108.79 | Source 3 |  |
| Note 1. Capacity in method 1 equals the minimal value of the DL and UL system capacity results Note 2. - 119dB for DDDUU Note 3. - 119dB for DDDUU |  |  |  |  |  |  |  |

Observations

- In Coverage Evaluation Method 1, FR1, DU, DL, VR/AR30, #UE/cell = Capacity (7~12), it is observed by Source 18, Source 20, Source 15, and Source 19 that mean DL coverage is -121.38dB.

- In Coverage Evaluation Method 1, FR1, DU, DL, VR/AR45, #UE/cell = Capacity (5~7), it is observed by Source 20, Source 15, and Source 19 that mean DL coverage is -121.86dB.

- In Coverage Evaluation Method 1, FR1, DU, DL, CG30, #UE/cell = Capacity (8~14), it is observed by Source 20, Source 15, and Source 19 that mean DL coverage is -122.09dB.

- In Coverage Evaluation Method 1, FR1, DU, UL, Pose, #UE/cell = 25, it is observed by Source 20 that mean UL coverage is -119.72dB.

- In Coverage Evaluation Method 1, FR1, DU, UL, AR1 stream, #UE/cell = Capacity (9~10), it is observed by Source 18, and Source 20 that mean UL coverage is -118.53dB.

Observations

- In Coverage Evaluation Method 1, FR1, DU, DL, VR/AR30, # UE/cell =1, it is observed by Source 18, Source 15, Source 7, Source 19, and Source 3 that mean DL coverage is -120.80dB.

- In Coverage Evaluation Method 1, FR1, DU, DL, VR/AR45, # UE/cell =1, it is observed by Source 15 and Source 19 that mean DL coverage is -121.93dB.

- In Coverage Evaluation Method 1, FR1, DU, DL, CG30, # UE/cell =1, it is observed by Source 15, Source 7, and Source 19 that mean DL coverage is -121.95dB.

- In Coverage Evaluation Method 1, FR1, DU, UL, Pose, #UE/cell =1, it is observed by Source 7 that mean UL coverage is -119dB.

- In Coverage Evaluation Method 1, FR1, DU, UL, AR1 stream, #UE/cell =1, it is observed by Source 18, Source 7, and Source 3 that mean UL coverage is -116.56dB.

Observations

- In Coverage Evaluation Method 1, FR1, DU, it was observed by the Source 7 that mean coverage of UL Pose is -119dB for both DDDSU and DDDUU.

- In Coverage Evaluation Method 1, FR1, DU, it was observed by the Source 7 that mean coverage of UL AR 1 stream is -118 and -119dB for DDDSU and DDDUU respectively.

##### UMa

Following table captures the company reported XR coverage evaluation results for FR1, UMa.

Table 141 XR Coverage results in Method 1, FR1, UMa, DDDSU

| Link direction | Applications | PDB (ms) | #UE/cell | XR Coverage |  | source | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Mean (dB) | Value (dB) |  |  |
| DL | VR/AR30 | 10 | 9 (Capacity) | -136.81 | -132.86 | Source 9 |  |
|  |  |  | 10 (Capacity) |  | -140.76 | Source 20 |  |
|  |  |  | 1 | -138.46 | -137.19 | Source 9 |  |
|  |  |  |  |  | -140.9 | Source 18 |  |
|  |  |  |  |  | -138 | Source 7 |  |
|  |  |  |  |  | -137.73 | Source 3 |  |
|  | VR/AR45 | 10 | 4 (Capacity) | -136.26 | -132.95 | Source 9 |  |
|  |  |  | 6 (Capacity) |  | -139.56 | Source 20 |  |
|  |  |  | 1 | -136.58 | -136.58 | Source 9 |  |
|  | CG30 | 15 | 12 (Capacity) | -137.12 | -134.38 | Source 9 |  |
|  |  |  | 11 (Capacity) |  | -139.86 | Source 20 |  |
|  |  |  | 1 | -137.59 | -137.19 | Source 9 |  |
|  |  |  |  |  | -138 | Source 7 |  |
| UL | Pose | 10 | 1 | -130.5 | -136.01 | Source 9 |  |
|  |  |  |  |  | -125 | Source 7 | Note 2 |
|  | AR 1 stream | 30 | 1 | -119.99 | -121.61 | Source 9 | Note 3 |
|  |  |  |  |  | -124.2 | Source 18 |  |
|  |  |  |  |  | -123 | Source 7 | Note 4 |
|  |  |  |  |  | -111.13 | Source 3 |  |
| Note 1. Capacity in method 1 equals the minimal value of the DL and UL system capacity results Note 2. -135dB for DDDUU Note 3. -118.91dB when PDB=15ms, -122.55dB when PDB=60ms Note 4. -127dB for DDDUU |  |  |  |  |  |  |  |

Observations

- In Coverage Evaluation Method 1, FR1, UMa, DL, VR/AR30, #UE/cell=Capacity(9~10), it is observed by Source 9 and Source 20 that mean DL coverage is -136.81dB.

- In Coverage Evaluation Method 1, FR1, UMa,  DL, VR/AR45, #UE/cell=Capacity(4~6), it is observed by Source 9 and Source 20 that mean DL coverage is -136.26dB.

- In Coverage Evaluation Method 1, FR1, UMa,  DL, CG30, #UE/cell=Capacity(11~12), it is observed by Source 9 and Source 20 that mean DL coverage is -137.12dB.

Observations

- In Coverage Evaluation Method 1, FR1, UMa, DL, VR/AR30,  #UE/cell =1, it is observed by Source 9, Source 18, Source 7, and Source 3 that mean DL coverage is -138.46dB.

- In Coverage Evaluation Method 1, FR1, UMa,  DL, VR/AR45, #UE/cell =1, it is observed by Source 9 that mean DL coverage is -136.58dB.

- In Coverage Evaluation Method 1, FR1, UMa,  DL, CG30 , #UE/cell =1, it is observed by Source 9 and  Source 7 that mean DL coverage is -137.59dB.

- In Coverage Evaluation Method 1, FR1, UMa, UL, Pose, #UE/cell =1, it is observed by Source 9 and Source 7 that mean UL coverage is -130.5dB.

- In Coverage Evaluation Method 1, FR1, UMa, UL, AR1 stream, #UE/cell =1, it is observed by Source 9, Source 18, Source 7, and Source 3 that mean UL coverage is -119.99dB.

Observations

- In Coverage Evaluation Method 1, FR1, UMa, #UE/cell =1, it was observed by Source 7 that the mean coverage of UL Pose with DDDUU is 10dB better than that with DDDSU.

- In Coverage Evaluation Method 1, FR1, UMa, # UE/cell =1, it was observed by Source 7 that the mean coverage of UL AR 1 stream with DDDUU is 4dB better than that with DDDSU.

- In Coverage Evaluation Method 1, FR1, UMa, # UE/cell =1, it was observed by Source 9 that the mean coverage of UL AR 1 stream for PDB=15ms, 30ms, and 60ms are given as -118.91dB, -121.61dB, and -122.55dB respectively.

##### InH

Following table captures the company reported XR coverage evaluation results for FR1, InH.

Table 142 XR Coverage results in Method 1, FR1, InH, DDDSU

| Link direction | Applications | PDB (ms) | #UE/cell | XR Coverage |  | source |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Mean (dB) | Value (dB) |  |
| DL | VR/AR30 | 10 | 11 (Capacity) | -76.62 | -82.24 | Source 20 |
|  |  |  | 5 (Capacity) |  | -71 | Source 15 |
|  |  |  | 1 | -72 | -72 | Source 15 |
|  | VR/AR45 | 10 | 7 (Capacity) | -76.06 | -82.12 | Source 20 |
|  |  |  | 3 (Capacity) |  | -70 | Source 15 |
|  |  |  | 1 | -71 | -71 | Source 15 |
|  | CG30 | 15 | 12 (Capacity) | -76.59 | -82.18 | Source 20 |
|  |  |  | 5 (Capacity) |  | -71 | Source 15 |
|  |  |  | 1 | -72 | -72 | Source 15 |
| UL | Pose | 10 | 25 | -79.62 | -79.62 | Source 20 |
|  | AR 1 stream | 30 | 10 | -79.56 | -79.56 | Source 20 |

Observations

- In Coverage Evaluation Method 1, FR1, InH,  DL. VR/AR30, #UE=Capacity(5~11), it is observed by Source 20 and Source 15 that mean DL coverage is -76.62dB.

- In Coverage Evaluation Method 1, FR1, InH,  DL, VR/AR45, #UE/cell=Capacity(3~7), it is observed by Source 20 and Source 15 that mean DL coverage is -76.06dB.

- In Coverage Evaluation Method 1, FR1, InH,  DL, CG30, #UE/cell=Capacity(5~12), it is observed by Source 20 and Source 15 that mean DL coverage is -76.59dB.

- In Coverage Evaluation Method 1, FR1, InH, UL, Pose, #UE/cell=25, it is observed by Source 20 that mean UL coverage for Pose  is -79.62dB

- In Coverage Evaluation Method 1, FR1, InH, UL, AR1 stream, #UE/cell=10, it is observed by Source 20 that mean UL coverage is -79.56dB.

Observations

- In Coverage Evaluation Method 1, FR1, InH, DL, VR/AR30, #UE/cell =1, it is observed by Source 15 that mean DL coverage is -72dB.

- In Coverage Evaluation Method 1, FR1, InH, DL, VR/AR45, #UE/cell =1, it is observed by Source 15 that mean DL coverage is -71dB.

- In Coverage Evaluation Method 1, FR1, InH, DL, CG30, #UE/cell =1, it is observed by Source 15 that mean DL coverage is -72dB.

#### FR2

##### DU

Following table captures the company reported XR coverage evaluation results for FR2, DU.

Table 143 XR Coverage results in Method 1, FR2, DU

| Link direction | Applications | PDB (ms) | #UE/cell | XR Coverage |  | TDD format | source |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Mean (dB) | Value (dB) |  |  |
| DL | VR/AR30 | 10 | 8 (Capacity) | -104.9 | -104.5 | DDDSU | Source 18 |
|  |  |  | 7 (Capacity) |  | -105.3 | DDDSU | Source 16 |
|  |  |  | 1 | -106.9 | -106.9 | DDDSU | Source 18 |
|  | CG8 | 15 | 32 (Capacity) | -105 | -105 | DDDSU | Source 16 |
| UL | Pose | 10 | 20 (Capacity) | -105.2 | -105.2 | DDDUU | Source 16 |
|  | AR 1 stream | 30 | 8 (Capacity) | -103.35 | -101.9 | DDDSU | Source 18 |
|  |  |  | 9 (Capacity) |  | -104.8 | DDDUU | Source 16 |
|  |  |  | 1 | -106.9 | -106.9 | DDDSU | Source 18 |

Observations

- In Coverage Evaluation Method 1, FR2, DU,  DL, VR/AR30, #UE/cell=Capacity (7~8), it is observed by Source 18 and Source 16 that mean DL coverage is -104.9dB.

- In Coverage Evaluation Method 1, FR2, DU,  DL, CG8, #UE/cell=Capacity (32), it is observed by Source 16 that mean DL coverage is -105dB.

- In Coverage Evaluation Method 1, FR2, DU,  UL, Pose, #UE/cell=Capacity(20) , it is observed by Source 16 that mean UL coverage for Pose is -105.2dB.

- In Coverage Evaluation Method 1, FR2, DU,  UL, AR 1 stream, #UE/cell=Capacity(8~9) , it is observed by Source 18 and Source 16 that mean UL coverage is -103.35dB.

- In Coverage Evaluation Method 1, FR2, DU,  DL, VR/AR30, #UE/cell =1, it is observed by Source 18 that mean DL coverage is -106.9dB.

- In Coverage Evaluation Method 1, FR2, DU, UL, AR 1 stream,  #UE/cell =1, it is observed by Source 18 that mean UL coverage is -106.9dB.

Observations

- In Coverage Evaluation Method 1, FR2, DU, AR30, #UE/cell=1, it is observed by Source 18 that the DL coverage is similar with that of UL.

##### InH

Following table captures the company reported XR coverage evaluation results for FR2, InH.

Table 144 XR Coverage results in Method 1, FR2, InH

| Link direction | Applications | PDB (ms) | #UE/cell | XR Coverage |  | TDD format | source |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Mean (dB) | Value (dB) |  |  |
| DL | VR/AR30 | 10 | 7(Capacity) | -86.5 | -86.5 | DDDSU | Source 16 |
|  | CG8 | 15 | 31(Capacity) | -85 | -85 | DDDSU | Source 16 |
| UL | Pose | 10 | 19(Capacity) | -84.5 | -84.5 | DDDUU | Source 16 |
|  | AR 1 stream | 30 | 10(Capacity) | -80.1 | -80.1 | DDDUU | Source 16 |

Observations

- In Coverage Evaluation Method 1, FR2, InH,  DL, VR/AR30, #UE/cell=Capacity(7), DDDSU, it is observed by Source 16 that mean DL coverage is -86.5dB.

- In Coverage Evaluation Method 1, FR2, InH,  DL, CG8, #UE/cell=Capacity(31), DDDSU, it is observed by Source 16 that mean DL coverage is -85dB.

- In Coverage Evaluation Method 1, FR2, InH,  UL, Pose, #UE/cell=Capacity(19), DDDUU, it is observed by Source 16 that mean UL coverage is -84.5dB.

- In Coverage Evaluation Method 1, FR2, InH,  UL, AR 1 stream, #UE/cell=Capacity(10), DDDUU, it is observed by Source 16 that mean UL coverage is -80.1dB.

### Coverage based on Methodology 2

In methodology 2, we evaluate XR coverage with 1 UE per network.

#### FR1

##### DU

Following table captures the company reported XR coverage evaluation results for FR1, DU.

Table 145 XR Coverage results in Method 2, FR1, DU, DDDSU

| Link direction | Applications | PDB (ms) | XR Coverage |  | source |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Mean (dB) | Value (dB) |  |
| DL | VR/AR30 | 10 | -139.89 | -144.58 | Source 18 |
|  |  |  |  | -134.80 | Source 10 |
|  |  |  |  | -137.4 | Source 16 |
|  |  |  |  | -142.78 | Source 3 |
|  | CG30 | 15 | -138.45 | -135.5 | Source 10 |
|  |  |  |  | -141.4 | Source 16 |
| UL | Pose | 10 | -137.45 | -134.6 | Source 10 |
|  |  |  |  | -140.3 | Source 16 |
|  | AR 1 stream | 30 | -118.99 | -126.84 | Source 18 |
|  |  |  |  | -111.13 | Source 3 |
|  | AR 2 streams | 10,30 | -119.9 | -119.9 | Source 16 |

Observations

- In Coverage Evaluation Method 2, FR1, DU, DL, VR/AR30, it is observed by Source 18, Source 10, Source 16, and Source 3 that mean DL coverage is -139.89dB.

- In Coverage Evaluation Method 2, FR1, DU, DL, CG30, it is observed by Source 10 and Source 16 that mean DL coverage is -138.45dB.

- In Coverage Evaluation Method 2, FR1, DU, UL, Pose, it is observed by Source 10 and Source 16 that mean UL coverage is -137.45 dB.

- In Coverage Evaluation Method 2, FR1, DU, UL, AR 1 stream, it is observed by Source 18 and Source 3 that mean UL coverage is -118.99 dB.

- In Coverage Evaluation Method 2, FR1, DU, UL, AR 2 stream, it is observed by Source 16 that mean UL coverage is -119.9dB.

##### UMa

Following table captures the company reported XR coverage evaluation results for FR1, UMa.

Table 146 XR Coverage results in Method 2, FR1, UMa, DDDSU

| Link direction | Applications | PDB (ms) | XR Coverage |  | source | Note |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Mean (dB) | Value (dB) |  |  |
| DL | VR/AR30 | 10 | -145.32 | -145.33 | Source 9 |  |
|  |  |  |  | -150.07 | Source 18 |  |
|  |  |  |  | -144.65 | Source 10 |  |
|  |  |  |  | -141.7 | Source 16 |  |
|  |  |  |  | -144.84 | Source 3 |  |
|  | VR/AR45 | 10 | -143.85 | -143.85 | Source 9 |  |
|  | CG30 | 15 | -147.16 | -146.88 | Source 9 |  |
|  |  |  |  | -148.2 | Source 10 |  |
|  |  |  |  | -146.4 | Source 16 |  |
| UL | Pose | 10 | -139.45 | -137.81 | Source 9 |  |
|  |  |  |  | -139.8 | Source 10 |  |
|  |  |  |  | -140.5 | Source 16 |  |
|  | AR 1 stream | 30 | -120.01 | -122.5 | Source 9 | Note 1 |
|  |  |  |  | -126.39 | Source 18 |  |
|  |  |  |  | -111.13 | Source 3 |  |
|  | AR 2 stream | 10,30 | -121.7 | -121.7 | Source 16 |  |
| Note 1. -119.85dB when PDB=15ms, -123.21dB for PDB=60ms |  |  |  |  |  |  |

Observations

- In Coverage Evaluation Method 2, FR1, UMa, DL, VR/AR30, it was observed by Source 9, Source 18, Source 10, Source 16 and Source 3 that the mean DL coverage is -145.32dB.

- In Coverage Evaluation Method 2, FR1, UMa, DL, VR/AR45, it was observed by Source 9 that the mean DL coverage of is -143.85dB.

- In Coverage Evaluation Method 2, FR1, UMa, DL, CG30, it was observed by Source 9, Source 10, and Source 16 that the mean DL coverage of is  -147.16dB.

- In Coverage Evaluation Method 2, FR1, UMa, UL, Pose, it was observed by Source 9, Source 10, and Source 16 that the mean UL coverage is -139.45dB.

- In Coverage Evaluation Method 2, FR1, UMa, UL, AR 1 stream, it was observed by Source 9, Source 18 and Source 3 that the mean UL coverage is -120.01dB.

- In Coverage Evaluation Method 2, FR1, UMa, UL, AR 2 streams, it was observed by Source 16 that the mean UL coverage is -121.7dB.

Observations

- For Coverage Evaluation Method 2, FR1, UMa, mean DL coverage of VR/CG/AR is better than its UL coverage.

- For Coverage Evaluation Method 2, FR1, UMa, applications with relaxed requirements (e.g., lower data rate, larger PDB) have larger coverage.

#### FR2

##### DU

Following table captures the company reported XR coverage evaluation results for FR2, DU.

Table 147 XR Coverage Results in Method 2, FR2, DU, DDDSU

| Link direction | Applications | PDB (ms) | XR Coverage (dB) |  | source |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Mean, | Data |  |
| DL | AR30 | 10 | -127.66 | -127.66 | Source 18 |
| UL | AR 1 stream | 30 | -120.17 | -120.17 | Source 18 |

Observations

- In Coverage Evaluation Method 2, FR2, DU, it was observed from Source 18 that mean DL coverage of AR30 is -127.66dB.

- In Coverage Evaluation Method 2, FR2, DU, it was observed from Source 18 that mean UL coverage of AR 1 stream is -120.17dB.

# XR Mobility evaluations

## Purpose of study

[As XR and Cloud Gaming see consumer adoption, the services are expected to be consumed by users on the move. Minimizing user experience degradation through mobility events is a key consideration in enabling mass adoption of such services. As such, mobility an important factor for XR and Cloud Gaming.]

## KPI

## Mobility evaluation results

This section is to capture the evaluation results and the corresponding observations for XR mobility. The following evaluation results and observations can be applied to both UL and DL traffic models for XR.

### Consecutive XR packets lost due to a HO event, N

The evaluation results of N are evaluated with variation of the frame generation rate of the flow in fps (F), packet delay budget in ms (PDB) and the HO interruption time in ms (Y), as summarized in Table 11.3.1-1.

Table 11.3.1-1. Summary of evaluation results of N

| F(fps) | PDB (ms) | Y (ms) | N (packets) | Source(s) | Note |
| --- | --- | --- | --- | --- | --- |
| 30 | 10 | 0~10 | 0~0.3 | Source 20 | Note 1 |
| 60 | 10 | 0~10 | 0~0.6 | Source 20 | Note 1 |
|  |  |  | 0 | Source 7, Source 9, Source 15 | Note 2 |
|  |  | 10~30 | 0.6~1 | Source 7, Source 9, Source 15 |  |
|  |  | 40~100 | 1.8~6 | Source 3, Source 7, Source 9, Source 10, Source 13, Source 14, Source 15, Source 17, Source 18 |  |
|  |  | 100~1000 | 7.9~51 | Source 10, Source 17 |  |
|  |  | 1000~4212 | 97~252 | Source 17 |  |
|  | 15 | 0~15 | 0 | Source 7, Source 15 | Note 2 |
|  |  | 15~30 | 0.3~1 | Source 7, Source 15 |  |
|  |  | 40~100 | 1.51~6 | Source 3, Source 6, Source 7, Source 13, Source 14, Source 15, Source 18 |  |
|  | 30 | 0~30 | 0 | Source 7, Source 15 | Note 2 |
|  |  | 40~100 | 0.6~5 | Source 7, Source 14, Source 15, Source 18 |  |
| 90 | 10 | 40~100 | 3.78~6.48 | Source 18 |  |
|  | 15 | 40~100 | 3.33~6.03 | Source 18 |  |
|  | 30 | 40~100 | 1.98~4.68 | Source 18 |  |
| 120 | 10 | 0~10 | 0~1.2 | Source 20 | Note 1 |
|  |  | 40~100 | 5.04~8.64 | Source 18 |  |
|  | 15 | 40~100 | 4.44~8.04 | Source 18 |  |
|  | 30 | 40~100 | 2.64~6.24 | Source 18 |  |
| 250 | 10 | 0~10 | 0 | Source 7, Source 15 | Note 2 |
|  |  | 10~30 | 2.5~3 | Source 7, Source 15 |  |
|  |  | 40~100 | 7.5~23 | Source 7, Source 13, Source 14, Source 15, Source 18 |  |
| Note 1: N = Y* F / 1000 + δ, Y < PDB, where δ = 0. Upper bound, assuming all the packets arriving within HO interruption time (Y) are lost Note 2: N = 0, Y < PDB. Lower bound, assuming all the packets arriving within HO interruption time are successfully received |  |  |  |  |  |

The followings are observed from the above Table 11.3.1-1,

- For XR application with F=30 FPS and PDB=10ms, with the range of Y 0~10ms, it is observed by Source 20 that the range of N is [0, 0.3) packets.

- For XR application with F=60 FPS and PDB=10ms, with the range of Y 0~10ms, it is observed by Source 20 that the range of N is [0, 0.6) packets.

- For XR application with F=60 FPS and PDB=10ms,

  - with the range of Y 0~10ms, it is observed by Source 7, Source 9, Source 15 that N is 0 packet.

  - with the range of Y 10~30ms, it is observed by Source 7, Source 9, Source 15 that the range of N is 0.6~1 packets.

  - With the range of Y 40~100ms, it is observed by Source 3, Source 7, Source 9, Source 10, Source 13, Source 14, Source 15, Source 17, Source 18 that the range of N is 1.8~6 packets.

  - With the range of Y 100~1000ms, it is observed by Source 10, Source 17 that the range of N is 7.9~51 packets.

  - With the range of Y 1000~4212ms, it is observed by Source 17 that the range of N is 97~252 packets.

- For XR application with F=60 FPS and PDB=15ms,

  - with the range of Y 0~15ms, it is observed by Source 7, Source 15 that N is 0 packet.

  - with the range of Y 15~30ms, it is observed by Source 7, Source 15 that the range of N is 0.3~1 packets.

  - with the range of Y 40~100ms, it is observed by Source 3, Source 6, Source 7, Source 13, Source 14, Source 15, Source 18 that the range of N is 1.51~6 packets.

- For XR application with F=60 FPS and PDB=30ms,

  - with the range of Y 0~30ms, it is observed by Source 7, Source 15 that N is 0 packet.

  - with the range of Y 40~100ms, it is observed by Source 7, Source 14, Source 15, Source 18 that the range of N is 0.6~5 packets.

- For XR application with F=90 FPS and PDB=10ms, with the range of Y 40~100ms, it is observed by Source 18 that the range of N is 3.78~6.48 packets.

- For XR application with F=90 FPS and PDB=15ms, with the range of Y 40~100ms, it is observed by Source 18 that the range of N is 3.33~6.03 packets.

- For XR application with F=90 FPS and PDB=30ms, with the range of Y 40~100ms, it is observed by Source 18 that the range of N is 1.98~4.68 packets.

- For XR application with F=120 FPS and PDB=10ms, with the range of Y 0~10ms, it is observed by Source 20 that the range of N is [0, 1.2) packets.

- For XR application with F=120 FPS and PDB=10ms, with the range of Y 0~100ms, it is observed by Source 18 that the range of N is 0~8.64 packets.

- For XR application with F=120 FPS and PDB=15ms, with the range of Y 40~100ms, it is observed by Source 18 that the range of N is 5.04~8.64 packets.

- For XR application with F=120 FPS and PDB=30ms, with the range of Y 40~100ms, it is observed by Source 18 that the range of N is 2.64~6.24 packets.

- For XR application with F=250 FPS and PDB=10ms, with the range of Y 0~10ms, it is observed by Source 7, Source 15 that N is 0 packet.

- For XR application with F=250 FPS and PDB=10ms, with the range of Y 10~30ms, it is observed by Source 7, Source 15 that the range of N is 2.5~3 packets.

- For XR application with F=250 FPS and PDB=10ms, with the range of Y 40~100ms, it is observed by  Source 7, Source 13, Source 14, Source 15, Source 18 that the range of N is 7.5~23 packets.

In summary, based on the evaluation results, it is identified that for XR applications:

- with a given FPS and a given PDB, N increases with the increase of Y.

- with a given PDB and a given Y, N increases with the increase of FPS.

- with a given FPS and a given Y, N decreases with the increase of PDB.

### Minimum target time interval between HO events, T

The evaluation results of T are evaluated with variation of packet success rate in % (X), packet error rate during time outside of handover procedure (PE,op), PDB and Y , as summarized in Table 11.3.2-1.

Table 11.3.2-1. Summary of evaluation results of T

| X | PE,op | PDB (ms) | Y (ms) | T (s) | Source(s) | Note |
| --- | --- | --- | --- | --- | --- | --- |
| 99% | 0% | 10 | 0~10 | 0 | Source 7, Source 9, Source 15 | Note 1 |
|  |  |  | 20 | 1 | Source 7, Source 9, Source 15 |  |
|  |  |  | 40~100 | 3~9 | Source 7, Source 9, Source 13, Source 14, Source 15, Source 18 |  |
|  |  | 15 | 0~15 | 0 | Source 7, Source 15 | Note 1 |
|  |  |  | 20 | 0.5 | Source 7, Source 15 |  |
|  |  |  | 40~100 | 2.5~8.5 | Source 7, Source 13, Source 14, Source 15, Source 18 |  |
|  |  | 30 | 0~30 | 0 | Source 7, Source 15 | Note 1 |
|  |  |  | 40~100 | 1~7 | Source 7, Source 13, Source 14, Source 15, Source 18 |  |
|  | 0.01%~0.4% | 10 | 0~10 | 0 | Source 7, Source 9, Source 15 | Note 1 |
|  |  |  | 0~10 | 0~1.66 | Source 20 | Note 2 |
|  |  |  | 20 | 1.16~1.25 | Source 7, Source 9, Source 15 |  |
|  |  |  | 30~100 | 2.22~14.94 | Source 3, Source 7, Source 9, Source 10, Source 13, Source 15, Source 17, Source 18, Source 20 |  |
|  |  |  | 100~1000 | 14.34~93 | Source 10, Source 17 |  |
|  |  |  | 1000~4212 | 164~466 | Source 17 |  |
|  |  | 15 | 0~15 | 0 | Source 7, Source 15 | Note 1 |
|  |  |  | 20 | 0.62~0.83 | Source 7, Source 15 |  |
|  |  |  | 40~100 | 3.1~14.11 | Source 3, Source 6, Source 7, Source 13, Source 15, Source 18 |  |
|  |  | 30 | 0~30 | 0 | Source 7, Source 15 | Note 1 |
|  |  |  | 40~100 | 1.2~11.62 | Source 7, Source 13, Source 15, Source 18 |  |
|  | 0.5%~0.7% | 10 | 0~10 | 0 | Source 7, Source 15 | Note 1 |
|  |  |  | 0~10 | 0~3.31 | Source 20 | Note 2 |
|  |  |  | 20 | 2.48 | Source 7, Source 15 |  |
|  |  |  | 30~100 | 3.98~22.4 | Source 7, Source 10, Source 13, Source 14, Source 15, Source 17, Source 18, Source 20 |  |
|  |  |  | 100~1000 | 28.83~168 | Source 10, Source 17 |  |
|  |  |  | 1000~4212 | 323~836 | Source 17 |  |
|  |  | 15 | 0~15 | 0 | Source 7, Source 15 | Note 1 |
|  |  |  | 20 | 1.24 | Source 7, Source 15 |  |
|  |  |  | 40~100 | 6.2~21.12 | Source 7, Source 13, Source 14, Source 15, Source 18 |  |
|  |  | 30 | 0~30 | 0 | Source 7, Source 15 | Note 1 |
|  |  |  | 40~100 | 2.48~17.4 | Source 7, Source 13, Source 14, Source 15, Source 18 |  |
|  | 0.8%~0.9% | 10 | 0~10 | 0 | Source 7, Source 15 | Note 1 |
|  |  |  | 0~10 | 0~9.91 | Source 20 | Note 2 |
|  |  |  | 20 | 4.96~9.91 | Source 7, Source 15 |  |
|  |  |  | 30~100 | 9.92~89.2 | Source 7, Source 10, Source 13, Source 14, Source 15, Source 18, Source 20 |  |
|  |  |  | 142 | 70.43~140.7 | Source 10 |  |
|  |  | 15 | 0~15 | 0 | Source 7, Source 15 | Note 1 |
|  |  |  | 20 | 2.48~4.95 | Source 7, Source 15 |  |
|  |  |  | 40~100 | 12.4~84.23 | Source 7, Source 13, Source 14, Source 15, Source 18 |  |
|  |  | 30 | 0~30 | 0 | Source 7, Source 15 | Note 1 |
|  |  |  | 40~100 | 4.96~69.4 | Source 7, Source 13, Source 14, Source 15, Source 18 |  |
| Note 1: T = 0, Y < PDB. Lower bound, assuming all the packets arriving within HO interruption time (Y) are successfully received Note 2: T = Y * (100%-PE,op) / (100%-X- PE,op), Y < PDB. Upper bound, assuming all the packets arriving within HO interruption time (Y) are lost. |  |  |  |  |  |  |

The followings are observed from the above Table 11.3.2-1,

- For XR application with X=99% and PDB=10ms, PE,op=0%,

  - with the range of Y 0~10ms, it is observed by Source 7, Source 9, Source 15 that T is 0 s.

  - with the Y=20ms, it is observed by Source 7, Source 9, Source 15 that T is 1 s.

  - with the range of Y 40~100ms, it is observed by Source 7, Source 9, Source 13, Source 14, Source 15, Source 18 that the range of T is 3~9 s.

- For XR application with X=99% and PDB=15ms, PE,op=0%,

  - with the range of Y 0~15ms, it is observed by Source 7, Source 15 that T is 0 s.

  - with the Y=20ms, it is observed by Source 7, Source 15 that T is 0.5 s.

  - with the range of Y 40~100ms, it is observed by Source 7, Source 13, Source 14, Source 15, Source 18 that the range of T is 2.5~8.5 s.

- For XR application with X=99% and PDB=30ms, PE,op=0%,

  - with the range of Y 0~30ms, it is observed by Source 7, Source 15 that T is 0 s.

  - with the range of Y 40~100ms, it is observed by Source 7, Source 13, Source 14, Source 15, Source 18 that the range of T is 1~7 s.

- For XR application with X=99% and PDB=10ms, the range of PE,op 0.01%~0.4%, with the range of Y 0~10ms, it is observed by Source 20 that the range of T is [0, 1.66) s.

- For XR application with X=99% and PDB=10ms, the range of PE,op 0.01%~0.4%,

  - with the range of Y 0~10ms, it is observed by Source 7, Source 9, Source 15 that T is 0 s.

  - with the Y=20ms, it is observed by Source 7, Source 9, Source 15 that the range of T is 1.16~1.25 s.

  - with the range of Y 30~100ms, it is observed by Source 3, Source 7, Source 9, Source 10, Source 13, Source 15, Source 17, Source 18, Source 20 that the range of T is 2.22~14.94 s.

- For XR application with X=99% and PDB=15ms, the range of PE,op 0.01%~0.4%,

  - with the range of Y 0~15ms, it is observed by Source 7, Source 15 that T is 0 s.

  - with the Y=20ms, it is observed by Source 7, Source 15 that the range of T is 0.62~0.83 s.

  - with the range of Y 40~100ms, it is observed by Source 3, Source 6, Source 7, Source 13, Source 15, Source 18 that the range of T is 3.1~14.11 s.

- For XR application with X=99% and PDB=30ms, the range of PE,op 0.01%~0.4%,

  - with the range of Y 0~30ms, it is observed by Source 7, Source 15 that T is 0 s.

  - with the range of Y 40~100ms, it is observed by Source 7, Source 13, Source 15, Source 18 that the range of T is 1.2~11.62 s.

- For XR application with X=99% and PDB=10ms, the range of PE,op 0.5%~0.7%, with the range of Y 0~10ms, it is observed by Source 20 that the range of T is [0, 3.31) s.

- For XR application with X=99% and PDB=10ms, the range of PE,op 0.5%~0.7%,

  - with the range of Y 0~10ms, it is observed by Source 7, Source 15 that T is 0 s.

  - with the Y=20ms, it is observed by Source 7, Source 15 that T is 2.48 s.

  - with the range of Y 30~100ms, it is observed by Source 7, Source 10, Source 13, Source 14, Source 15, Source 17, Source 18, Source 20 that the range of T is 3.98~22.4 s.

- For XR application with X=99% and PDB=15ms, the range of PE,op 0.5%~0.7%,

  - with the range of Y 0~15ms, it is observed by Source 7, Source 15 that T is 0 s.

  - with the Y=20ms, it is observed by Source 7, Source 15 that T is 1.24 s.

  - with the range of Y 40~100ms, it is observed by Source 7, Source 13, Source 14, Source 15, Source 18 that the range of T is 6.2~21.12 s.

- For XR application with X=99% and PDB=30ms, the range of PE,op 0.5%~0.7%,

  - with the range of Y 0~30ms, it is observed by Source 7, Source 15 that T is 0 s.

  - with the range of Y 40~100ms, it is observed by Source 7, Source 13, Source 14, Source 15, Source 18 that the range of T is 2.48~17.4 s.

- For XR application with X=99% and PDB=10ms, the range of PE,op 0.8%~0.9%, with the range of Y 0~10ms, it is observed by Source 20 that the range of T is [0, 9.91) s.

- For XR application with X=99% and PDB=10ms, the range of PE,op 0.8%~0.9%,

  - with the range of Y 0~10ms, it is observed by Source 7, Source 15 that T is 0 s.

  - with the Y=20ms, it is observed by Source 7, Source 15 that the range of T is 4.96~9.91 s.

  - with the range of Y 30~100ms, it is observed by Source 7, Source 10, Source 13, Source 14, Source 15, Source 18, Source 20 that the range of T is 9.92~89.2 s.

- For XR application with X=99% and PDB=15ms, the range of PE,op 0.8%~0.9%,

  - with the range of Y 0~15ms, it is observed by Source 7, Source 15 that T is 0 s.

  - with the Y=20ms, it is observed by Source 7, Source 15 that the range of T is 2.48~4.95 s.

  - with the range of Y 40~100ms, it is observed by Source 7, Source 13, Source 14, Source 15, Source 18 that the range of T is 12.4~84.23 s.

- For XR application with X=99% and PDB=30ms, the range of PE,op 0.8%~0.9%,

  - with the range of Y 0~30ms, it is observed by Source 7, Source 15 that T is 0 s.

  - with the range of Y 40~100ms, it is observed by Source 7, Source 13, Source 14, Source 15, Source 18 that the range of T is 4.96~69.4 s.

- It is observed by Source 20 that as PE,op /PER increases from 0.8 to 0.99, T/Y increases evidently (up to 20 times). This is insensitive to PER.

In summary, based on the evaluation results, it is identified that for XR applications and a given X:

- with a given PE,op and a given PDB, T increases with the increase of Y.

- with a given PE,op and a given Y, T decreases with the increase of PDB.

- with a given PDB and a given Y, with PE,op < 100%-X, T non-linearly increases with the increase of PE,op.

# Conclusions

The study focused on the following objectives: (1) confirm XR and Cloud Gaming (CG) applications of interest, (2) identify the traffic models for the applications of interest taking outcome of SA WG4 work as input, (3) identify evaluation methodology and KPIs to assess XR and CG performance for relevant deployment scenarios, (4) evaluate XR and CG performance towards characterization of identified KPIs.

Diverse AR, VR, and CG applications were identified and confirmed as the applications of interest in the study. These applications include, but not limited to: VR1 (Viewport dependent streaming), VR2 (Split Rendering: Viewport rendering with Time Warp in device), AR1 (XR Distributed Computing), AR2 (XR Conversational), and CG.

Traffic models and characteristics of AR, VR, and CG applications were developed taking into account NR RAN performance evaluations. The traffic models include single stream downlink (DL) traffic model for VR/AR/CG, optional multi-stream DL traffic model for VR/AR/CG, single stream uplink (UL) traffic models for VR/AR/CG, and multi-stream UL traffic model for AR, as described in Clause 6. A baseline per UE KPI which considers PER and PDB is identified and used for subsequent evaluations.

The AR, VR, and CG performance for NR was evaluated using the traffic models for FR1 and FR2 in various deployment scenarios (indoor hotspot, dense urban, and urban macro) in terms of capacity, UE power consumption, coverage and mobility.

XR capacity

The capacity for AR, VR, and CG applications was evaluated and the results are summarized as follows:

- The baseline capacity for AR, VR, and CG in FR1 DL/UL and FR2 DL/UL were evaluated based on the agreed traffic models, evaluation methodology, and KPIs, with the results collected in Clause 8.3.1. The evaluation results show that

    - 5G NR can support AR, VR, and CG for the evaluated cases and scenarios, where the capacity in urban macro scenario is generally lower than that in dense urban and indoor hotspot scenarios, in particular for AR applications with uplink video.

- The capacity impact of different data-rates, different PDB/PER (packet delay budget/packet error rate) values, jitter, dual-eye buffer staggering, different TDD frame formats, different bandwidths, or FDM/SDM and mini-slot operations have been evaluated.  The results and observations are given in Clause 8.3.2. Based on the evaluation results, the following is observed:

  - The NR system capacity in support of AR, VR, and CG applications is smaller for applications requiring higher data rate.

  - The NR system capacity in support of AR, VR, and CG applications is higher with larger PDB value and/or relaxed PER requirement (i.e., higher PER values).

  - The AR, VR, and CG capacity is higher with larger system bandwidth.

- Various potential NR capacity enhancement schemes in support of XR services were proposed and evaluated by different companies.  Their results are collected in Clause 8.3.3.

XR UE power consumption

The UE power consumption for AR, VR, and CG applications was evaluated and the results are summarized as follows:

- The power saving gain from Release 15, 16, and 17 power saving schemes including CDRX, PDCCH monitoring adaptation, cross slot scheduling, MIMO layer adaptation was evaluated with respect to the case when UE is always on, i.e., UE is available for gNB scheduling for all slots. Corresponding results and observations are given in Clause 9.3.1.

- The UE power consumption was evaluated for different parameters. The results are collected in Clause 9.3.2.  The following is observed from the results:

  - There is a trade-off between UE power saving gain and capacity.

  - The choice of a CDRX configuration (cycle, on duration, and inactivity timer) greatly affects the power saving (PS) gain.

- The potential enhancement schemes for UE power saving were proposed and evaluated by different companies.  Their results and observations are given in Clause 9.3.3.

XR coverage

The AR, VR, and CG coverage was evaluated based on the agreed traffic model and two methodologies for coverage evaluation. Note that these two methodologies are different from the traditional methodology based on link budget for coverage evaluation. The results are collected in Clause 10.3.

According to the evaluation results, it is observed that for deployment scenarios of dense urban and urban macro, UL coverage is worse than DL coverage.

XR mobility

The performance of mobility for AR, VR, and CG applications was studied. The study considers two mobility KPIs given in Clause 11.2: number of consecutive XR packets lost due to a handover event and minimum target time between handover events. The evaluation methodology of mobility performance is a simplified analytical approach given in Clause A.4, and the evaluation results are collected in Clause 11.3. The following is observed from the results:

  - Higher PDB leads to lower (better) mobility KPIs.

  - Higher frame rate leads to higher (worse) number of consecutive XR packets lost.

  - When varying the handover interruption time, the lower (better) mobility KPIs are achieved when handover interruption time is lower than PDB.

  - Higher handover interruption time leads to higher (worse) mobility KPIs.

Based on the study, it is recommended to further study and enhance at least the capacity and UE power consumption performance of 5G NR for XR and CG applications.

# Annex <A>: Evaluation Methodology

# A.1	Evaluation Methodology for Capacity

System Level Simulation Parameters for Capacity Evaluation

For Capacity evaluation, system level simulation (SLS) is carried out based on the simulation parameters presented in Error! Not a valid bookmark self-reference. (for FR1) and Table 3 (for FR2) should be used.

The gNB and UEs in the simulation are configured base on  simulation parameters presented in Error! Not a valid bookmark self-reference. (for FR1) and Table 3 (for FR2) and the traffic model used for the simulation is selected from the XR/CG traffic models presented in Section 6.

DL-only and UL-only Evaluation

In capacity evaluation, the DL and UL evaluation is done separately and independently.

UE Dropping

For a given number of UEs per cell, N, the N UEs are randomly dropped in the network using the UE distribution specified in Error! Not a valid bookmark self-reference. (for FR1) and Table 3 (for FR2) for the chosen deployment scenario. Either exactly equal number of UEs per cell could be assumed or on average N Ues per cell could be assumed. Either approach is accepted, and companies are to report the method used for their evaluation.

Packet Discarding

Once communication commence between the UE and gNB, an XR/CG packet is deemed in error (i.e., lost) when it has exceeded the PDB, such that it will be added to the PER counting. It is up to company to report the details for the handling of packet which has exceeded the PDB, e.g.

- Option 1: The packet exceeding the delay is still delivered to the other side

- Option 2: The packet (including the non-transmitted part) is discarded at the transmitter (at the gNB for DL packets and at the UE for UL packets)

- Other options are not precluded

Satisfied UE and Capacity

For a given UE, the achieved PER for all packets communicated during the session is determined. Using the achieved PER per UE, the percentage of the satisfied UEs can be determined for this simulation.

Multiple runs of the SLS are required to sweep the number of UEs per cell, N, in order to determine, the capacity C (i.e., the maximum value of N satisfying at least 90% of the UEs are satisfied (see Section 8.2 for definition of capacity))

The system capacity for DL and UL are identified separately though independent evaluation.

Additional Metrics

In addition to the KPIs discussed in Section 8.2, following performance metrics can be optionally reported.

- Percentage of satisfied UEs

- CDF of packet error ratio

- CDF of packet latency

- CDF of user-perceived throughput

- Resource utilization

Table 220: System Simulation Parameters for FR1

| Parameter | Deployment scenarios |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Dense Urban (38.913 w/ following parameters) | Urban Macro (38.913 w/ following parameters) |  | Indoor Hotspot (38.913 w/ following parameters) |  |
| Layout | 21cells with wraparound ISD: 200m | 21cells with wraparound ISD = 500 m |  | 120m x 50m ISD: 20m TRP numbers: 12 |  |
| Channel model | UMa (38.901) | UMa (38.901) |  | InH(38.901) |  |
| UE Distribution | 80% indoor, 20% outdoor Note: Other UE distribution can be evaluated optionally. |  |  | 100% indoor |  |
| Carrier frequency | 4 GHz |  |  |  |  |
| Subcarrier spacing | 30 kHz |  |  |  |  |
| BS height | 25m | 25m |  | 3m |  |
| UE height | For Dense urban and Urban Macro, the UE height for indoor UEs is updated as following based on Table 6-1 in TR 36.873. UE height (hUT) in meters; general equation for UE height; hUT=3(nfl – 1) + 1.5. nfl for outdoor UEs; 1. nfl for indoor UEs; nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8). | 1.5 m |  |
| BS noise figure | 5 dB |  |  |  |  |
| UE noise figure | 9 dB |  |  |  |  |
| BS receiver | MMSE-IRC |  |  |  |  |
| UE receiver | MMSE-IRC |  |  |  |  |
| Channel estimation | Realistic Ideal(optional) |  |  |  |  |
| UE speed | 3 km/hr |  |  |  |  |
| MCS | Up to 256QAM |  |  |  |  |
| BS Antenna Pattern | 3-sector antenna radiation pattern, 8 dBi |  |  | Ceiling-mount antenna radiation pattern, 5 dBi |  |
| BS Antenna Configuration | Option 1: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Option 2: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1,8,2) (dH, dV) = (0.5λ, 0.5λ) Note: Other BS antenna parameters can also be optionally evaluated. |  |  | 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (4,4,2,1,1;4,4) (dH, dV) = (0.5λ, 0.5λ) Note: Other BS antenna parameters can also be optionally evaluated. |  |
| UE Antenna Pattern | Omni-directional, 0 dBi, |  |  |  |  |
| UE Antenna Configuration | Baseline: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ Optional: 4T/4R, 1T/2R, 2T2R |  |  |  |  |
| Down Tilt | 12 degrees Note: Other downtilt values can also be optionally evaluated |  | Up to company report |  | 90° (pointing to the ground) Note: Other downtilt values can also be optionally evaluated |
| BS Transmit Power | 44 dBm per 20 MHz Note: For system BW larger than above, Tx power scales up accordingly. |  | 49 dBm/20 MHz |  | 24 dBm per 20 MHz Note: For system BW larger than above, Tx power scales up accordingly. |
| UE max tx power | 23dBm |  |  |  |  |
| System Bandwidth | Single Carrier (SC) evaluations, - Baseline: 100 MHz - Optional: 20/40 MHz, CA evaluations, - Optional: 2*100 MHz with CA Note: Other system bandwidths can also be optionally evaluated |  |  |  |  |

Table 3: System Simulation Parameters for FR2

| Parameter | Deployment scenarios |  |
| --- | --- | --- |
|  | Dense Urban (38.913 w/ following parameters) | Indoor Hotspot (38.913 w/ following parameters) |
| Layout | 21cells with wraparound ISD: 200m | 120m x 50m ISD: 20m TRP numbers: 12 |
| Channel Model | UMa(38.901) | InH(38.901) |
| UE Distribution | For indoor scenario: 100% indoor For outdoor scenario: 100% outdoor Note: Other UE distribution can be evaluated optionally. |  |
| Carrier frequency | 30 GHz |  |
| Subcarrier spacing | 120 KHz |  |
| BS height | 25m | 3m |
| UE height | The UE height for indoor UEs is updated as following based on Table 6-1 in TR 36.873. UE height (hUT) in meters; general equation for UE height; hUT=3(nfl – 1) + 1.5. nfl for outdoor UEs; 1. nfl for indoor UEs; nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8). | 1.5m |
| BS noise figure | 7 dB |  |
| UE noise figure | 13 dB |  |
| BS receiver | MMSE-IRC |  |
| UE receiver | MMSE-IRC |  |
| UE speed | 3 km/hr |  |
| MCS | Up to 256QAM |  |
| BS antenna pattern | 3-sector antenna radiation pattern, 8 dBi | Ceiling-mount antenna radiation pattern, 5 dBi |
| BS Antenna Configuration | 2 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (4,8,2,2,2;1,1) (dH, dV) = (0.5λ, 0.5λ) Note: Other BS antenna parameters can also be optionally evaluated. | 2 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (16, 8, 2,1,1;1,1) (dH, dV) = (0.5, 0.5)λ Note: Other BS antenna parameters can also be optionally evaluated. |
| UE Antenna Pattern | UE antenna radiation pattern model 1, 5dBi |  |
| UE Antenna Configuration | Option 1: (Follow Rel-17 evaluation methodology for FeMIMO in R1-2007151) (M, N, P)=(1, 4, 2), 3 panels (left, right, top) Option 2: (from TR 38.802 – developed in Rel-14) 4Tx/4Rx: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2), (dH,dV) = (0.5, 0.5)λ, the polarization angles are 0° and 90° Note: Other UE antenna parameters can also be optionally evaluated. |  |
| Downtilt | Not specified | 90° (pointing to the ground) Note: Other downtilt values can also be optionally evaluated |
| BS Transmit Power | 40 dBm per 80 MHz. EIRP should not exceed 73 dBm Note: For system BW larger than above, Tx power scales up accordingly. | 23 dBm per 80 MHz. EIRP should not exceed 58 dBm Note: For system BW larger than above, Tx power scales up accordingly. |
| UE max tx power | 23dBm, maximum EIRP 43 dBm |  |
| System Bandwidth | Single Carrier Evaluations: - Option 1: 100 MHz - Option 2: 400 MHz CA Evaluations: - Companies should report the CA system bandwidth if CA is configured. Note: Other system bandwidths can also be optionally evaluated |  |

Table 222 includes common assumptions applied to both FR1 and FR2.

Table 222 Common Assumptions for FR1 and FR2

| TDD Configuration | Option 1: DDDSU Option 2: DDDUU Note: Detailed S slot format is 10D:2F:2U. Note: For option 2, there is a 2-symbol gap at the end to third “D” slot of DDDUU |
| --- | --- |
| Scheduler | SU/MU-MIMO PF scheduler (company to report SU or MU), other scheduler (e.g., delay aware scheduler) is up to companies report |
| Channel Estimation /CSI acquisition | Realistic Both CSI feedback and SRS are considered Companies should report • CSI feedback delay, CSI report periodicity, whether using CSI quantization, CSI error model or not, • Assumptions on SRS: periodicity, processing gain, processing delay, etc Note: Companies may optionally use ideal channel estimation |
| PHY processing delay | Baseline: UE PDSCH processing Capability #1 Optional: UE PDSCH processing Capability #2 Companies should report gNB processing delay, e.g. DL NACK to retransmission delay, UL previous transmission to current transmission delay and etc. |
| PDCCH overhead | Companies should report |
| DMRS overhead | Companies should report |
| Target BLER | Companies should report |
| Max HARQ transmission | Companies should report |
| Power control parameter | Companies should report |
| Transmission scheme | Companies should report |

# A.2	Evaluation Methodology for Power

Baseline UE Power Model

For XR UE power evaluation, the power model presented in [6] is used with additional modifications presented in this section.

System Level Power Evaluation

In this study, UE power consumption evaluation system is done in system level setup; UE are distributed across multiple cells are their dynamic DL rx and UL tx activities are considered in each UE’s power consumption evaluation including time varying channel conditions, and dynamic scheduling for DL and UL with HARQ operations, power control, etc. With system level setup, the study allows to capture distribution of UE power consumptions across different locations in the cell showing different power consumption distributions across different rx/tx physical channel activities (e.g., PDCCH, PDSCH, PUSCH, PUCCH, …).

More importantly, the system level power evaluation allows the joint evaluation of capacity and power – allowing to capture the interaction between scheduler and power saving mechanism (e.g., CDRX) revealing capacity and power tradeoff. Due to the power of system level power evaluation framework, one can compare different power saving mechanisms with capacity in consideration, i.e., making the comparison fair by making it subject to limited capacity loss.

Power Saving Schemes

To evaluate the power saving impact of different power saving schemes for XR/CG, companies are encouraged to evaluate UE power consumption as described below:

As a baseline, UE is always ON, i.e., UE is always available for gNB scheduling of XR/CG traffic, is considered. The power consumption of AlwaysOn could be reference for power saving gain calculation.

Optionally following power saving mechanisms can be further considered.

- R15/16 CDRX mechanism can be optionally evaluated. The CDRX configuration can be reported with the evaluation results.

- Release 15/16/17 Connected mode power saving techniques such as BWP switching, PDCCH skipping and search space switching can also be evaluated.

- Genie scheme can be studied. The Genie power saving scheme works such that UE is in a sleep state (e.g., micro/light/deep sleep as defined in TR38.840) whenever there is neither DL data reception nor UL transmission. From the gNB scheduling perspective, it is assumed that UE is always available for scheduling, i.e., there is no difference from Baseline in gNB scheduling and corresponding UE transmission/reception availability. Note that the Genie approach is expected to provide an upper bound of power savings gain since the UE is able to take advantage of all the unscheduled slot duration by entering sleep state whenever possible.

- [Other schemes not listed here could be also evaluated.]

DL and UL Power Saving Evaluation

For XR/CG power consumption evaluation, DL and UL power consumption can be evaluated based on following methods.

- DL-only Evaluation, UL-only Evaluation: DL and UL power are evaluated separately and independently, and the DL and UL power consumption results are collected separately.

- DL+UL Joint Evaluation: DL and UL performances are evaluated together, and DL and UL power consumption are counted to obtain the total power consumption.

Note that adding DL-only power number and UL-only power will not give the equal power number as DL+UL joint power number due to duplicate power cost counted in DL / UL-only method such as PDCCH monitoring.

System-level Simulation Flow

For XR UE power consumption evaluation, SLS is carried out using the capacity evaluation methodology for the baseline and power saving scheme under investigation. The details are presented below:

- Step 1) Determine a scenario/application/configurations/power saving schemes for evaluation.

- Step 2) Determination of the number of Ues per cell N for power evaluation.

  - N is set to floor(C), where C is the XR capacity for the given scenario/application/configuration and floor() is flooring operation.

  - Note that N=floor(C) corresponds to the high system load case.

  - Optionally, N could be set to smaller than C (N<< C) for low load case.

- Step 3) Perform system level power evaluation for the given scenario/application/configuration and N determined in Step 2)

- Step 4) Following metrics are reported.

  - satisfied UE rate in capacity evaluation

  - satisfied UE rate in power evaluation

  - PSG CDF 5, 50, 95% points

    - Power saving gain is computed w.r.t to AlwaysOn case.

    - PSG could be computed w.r.t either all UE or satisfied Ues. Companies to report method used with their results.

Additional UL Power Modelling

One of necessary enhancements of power model for system level power evaluation is the UL power consumption model. The UL power model in TR 38.840 is incomplete for the case of UE transmit power other than 0 and 23dBm. The power consumption values corresponding to UE transmit powers other than 0 dBm and 23 dBm (cell middle UEs) are not defined. Therefore, to determine the power consumption for such scenarios, companies are encouraged to use the following methods to estimate UE power consumptions when transmitting with tx power other than 0 and 23dBm:

Linear interpolation method in linear scale for Tx power values other than 0 dBm and 23 dBm. Companies are to indicate how they do linear interpolation method in linear scale considering step-wise linear average of UE power model.

As another method that can be used for evaluation, consider only two Tx power values as defined in TR 38.840.  Power number is given as A for X= [0, M]dBm and B for X =[M, 23]dBm, where A and B (defined in 38.840) correspond to power consumption numbers for a given uplink slot for 0dBm and 23dBm respectively with M = [20] or other value(s).

The power consumption of the UE transmitting with power less than 0 dBm could be set to the power number of 0 dBm. Alternatively, companies could choose to adopt the extrapolation of the power numbers from on 0 and 23dBm power numbers.

For other missing UL power modelling, companies to use their own model and report with their results.

# A.3	Evaluation Methodology for Coverage

For XR/CG Coverage Evaluation, there are two options for evaluating the coverage based on the coupling gain metric. The coupling gain is defined as the ratio of received and transmitted power measured in dB, and includes antenna gains, path loss, shadowing, indoor- or body loss, etc. For more information about coupling gain, readers are referred to TR 37.910.

Below are the two methodologies:

Coverage Evaluation Methodology 1

For a given XR application (AR/VR/CG) in a given deployment scenario (DU/InH/UMa), the XR/CG in DL or UL coverage is determined as follows:

- Run SLS with #UEs per cell = 1 as shown in Figure 2 and/or XR/CG capacity using the XR system capacity evaluation methodology presented in A.1.

- Determine the “satisfied UE” and evaluate coupling gain for those UEs.

- The coverage is defined to be the 5-percentile point in CDF of coupling gain for the “satisfied” Ues.

Note: For this methodology, the evaluation of coupling gain will be impacted by e.g., interference and scheduler mechanism, etc.

Figure 2 Layout and UE distribution in Methodology 1 (1 UE per cell)

Coverage Evaluation Methodology 2

For a given XR application (AR/VR/CG) for a given deployment scenario (DU/InH/UMa), the XR/CG in DL or UL coverage is determined as follows:

- Run SLS with #UEs per cell = 1 as shown in Figure 3. The UE is randomly dropped in the entire network (or in all the cells) that is associated with one of the three center cells (or gNBs), i.e., only one of the center gNBs is activated.

- Run SLS according to capacity evaluation methodology and determine whether the UE is satisfied or not.

- The coverage is defined to be the 5-percentile point in the CDF curve of coupling gain for all the satisfied Ues.

Figure 3 Layout and UE distribution in Methodology 2 (1 UE / network)

# A.4	Evaluation Methodology for Mobility

For XR/CG mobility evaluation, mobility performance is evaluated analytically while taking into account mobility procedures, traffic models, and user satisfaction criteria.

The mobility performance metrics {N, T}, where N is the number of consecutive XR packets lost due to a HO event and T is the minimum target time interval between HO events in ms, are obtained by the following steps:

- Step1. Calculate HO interruption time in ms (Y) for existing HO techniques by directly following the requirements given in TS 38.133.

- Step2. For a Y and the XR traffic pattern characterized by the packet arrival rate in average as determined by the frame generation rate of the flow in fps (F) in 6.1.1.2, and the packet delay budget in ms (PDB):

  - N is estimated as:

N=(Y-PDB)×F1000   Y≥PDB

  - T is estimated as:

T=(Y-PDB)×100%-PE,op100%-X-PE,op   Y≥PDB

    - where PE,op is packet error rate during time outside of handover procedure and X is packet success rate (baseline: X = 99%, other X value(s) can be also evaluated).

  - The case of Y < PDB can be optionally evaluated, e,g.

N=0, or N=Y×F1000    Y<PDB

T=0, or T=Y×100%-PE,op100%-X-PE,op      Y<PDB

Note 1: mobility evaluation is performed in Dense Urban and Urban Macro.

Note 2: T maybe affected by system load, interference, etc.

# Annex <B> Source Specific Capacity Performance Evaluation Results

  - FR1 DL

    - DU Scenario

      - VR/AR

        - Single stream traffic model

Table B.1.1.1.1-1. FR1, DL, DU, VR/AR 30Mbps, 60FPS, SU-MIMO, 100Mbps bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 9 | R1-2110811 | DDDSU | SU-MIMO | Close loop rank adaptation | random | 10 | 5.1 | 5 | 91.43% | Note 1 |
| Source 9 | R1-2110811 | DDDSU | SU-MIMO | Close loop rank adaptation | random | 10 | 6.4 | 6 | 91.67% | Note 1, 3 |
| Source 8 | R1-2110885 | DDDUU | SU-MIMO | Zeroforcing | random | 10 | 7.6 | 7 | 92% | Note 1 |
| Source 8 | R1-2110885 | DDDUU | SU-MIMO | cooperative MIMO/precoding | random | 10 | 9.4 | 9 | 93% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | SU-MIMO | Zeroforcing | random | 10 | 9.7 | 9 | 94% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | SU-MIMO | cooperative MIMO/precoding | random | 10 | 11.7 | 11 | 92% | Note 1 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 9.49 | 9 | 94.18% | Note 1 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 12.67 | 12 | 95.12% | Note 1, 4 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 13.47 | 13 | 94.05% | Note 1, 5 |
| Source 3 | R1-2111234 | DDDSU | SU-MIMO | codebook-based Type 2 | random | 10 | 8 | 8 | 91% | Note1, 7, 8 |
| Source 4 | R1-2108869 | DDDSU | SU-MIMO | reciprocity-based precoding | same | 10 | 4.05 | 4 | 90% | Note 2 |
| Source 6 | R1-2109307 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 1 | 1 | 95.24% | Note 1, 4 |
| Source 10 | R1-2111521 | DDDSU | SU-MIMO |  | random | 10 | 5.45 | 5 | 94.19% | Note 1, 10, 11 |
| Source 10 | R1-2111521 | DDDSU | SU-MIMO |  | random | 10 | 7.18 | 7 | 91.9% | Note 1, 11 |
| Source 10 | R1-2111521 | DDDSU | SU-MIMO |  | random | 10 | 5.7 | 5 | 94.76% | Note 1, 10, 12 |
| Source 10 | R1-2111521 | DDDSU | SU-MIMO |  | random | 10 | 7.31 | 7 | 93.19% | Note 1, 12 |
| Source 19 | R1-2112573 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 7 | 7 | 92.44% | Note 2 |
| Source 15 | R1-2112572 | DDDSU | SU-MIMO |  | random | 10 | 6.54 | 6 | 97% | Note 2 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 8.2 | 8 | 93% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | reciprocity-based precoding | All Sync | 10 | 7 | 7 | 90% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | reciprocity-based precoding | Random | 10 | 8.8 | 8 | 97% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | reciprocity-based precoding | Evenly Spaced | 10 | 9.1 | 9 | 93% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | reciprocity-based precoding | All Sync | 10 | 3.1 | 3 | 92% | Note 1, 6 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | reciprocity-based precoding | Random | 10 | 6.3 | 6 | 93% | Note 1, 6 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | reciprocity-based precoding | Evenly Spaced | 10 | 8.3 | 8 | 93% | Note 1, 6 |
| Source 5 | R1- 2112079 | DDDSU | SU-MIMO | reciprocity-based precoding | Random | 10 | 6.3 | 6 | 96..83% | Note 1 |
| Source 14 | R1-2112296 | DDDSU | SU-MIMO | codebook-based Type 2 | random | 10 | 10.6 | 10 | 94.30% | Note 1 |
| Source 17 | R1-2111349 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 8.4 | 8 | 95% | Note 2 |
| Source 17 | R1-2111349 | DDDSU | SU-MIMO | reciprocity-based precoding | evenly spaced | 10 | 9.2 | 9 | 91% | Note 2 |
| Source 17 | R1-2111349 | DDDSU | SU-MIMO | reciprocity-based precoding | same | 10 | 7.4 | 7 | 95% | Note 2 |
| Source 17 | R1-2111349 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 9 | 9 | 90% | Note 2, 9 |
| Source 17 | R1-2111349 | DDDSU | SU-MIMO | reciprocity-based precoding | evenly spaced | 10 | 10.5 | 10 | 94% | Note 2, 9 |
| Source 17 | R1-2111349 | DDDSU | SU-MIMO | reciprocity-based precoding | same | 10 | 7.1 | 7 | 92% | Note 2, 9 |
| Source 7 | R1-2110144 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 9.3 |  |  | Note 1 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: BS antenna parameters: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1:8,2) Note 3: DL scheduler for dynamic grant based PDSCH scheduling: Frame Level Integrated Transmission (FLIT) Note 4: DL scheduler for dynamic grant based PDSCH scheduling: Delay aware (DA) Note 5: stream packet generation rate (Fps or Hz): 120 Note 6: stream packet generation rate (Fps or Hz): 30 Note 7: 64QAM Note 8: Jitter STD=2ms, Jitter range Min=0ms, Jitter range Max=8ms Note 9: Without Jitter Note 10: Target BLER: 1% Note 11: Not discard packet not meeting PDB Note 12: Discard packet not meeting PDB |  |  |  |  |  |  |  |  |  |  |

Table B.1.1.1.1-2. FR1, DL, DU, VR/AR 30Mbps, 60FPS, MU-MIMO, 100Mbps bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | Close loop rank adaptation | random | 10 | 11.5 | 11 | 92.99% | Note 1 |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | Close loop rank adaptation | random | 10 | 9.9 | 9 | 94.36% | Note 1,4 |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | Close loop rank adaptation | random | 10 | 11.5 | 11 | 92.99% | Note 1 |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | Close loop rank adaptation | random | 10 | 16.8 | 16 | 91.96% | Note 1, 5 |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | Close loop rank adaptation | random | 7 | 6.3 | 6 | 91.67% | Note 1 |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | Close loop rank adaptation | random | 13 | 14.6 | 14 | 91.72% | Note 1 |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | Close loop rank adaptation | random | 13 | 19.3 | 19 | 90.54% | Note 1, 5 |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | Close loop rank adaptation | random | 10 | 11.6 | 11 | 93.42% | Note 1, 6 |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | Close loop rank adaptation | random | 10 | 14 | 14 | 90.08% | Note 1, 3 |
| Source 8 | R1-2110885 | DDDUU | MU-MIMO | Zeroforcing | random | 10 | 8.9 | 8 | 92% | Note 1 |
| Source 8 | R1-2110885 | DDDUU | MU-MIMO | cooperative MIMO/precoding | random | 10 | 16.4 | 16 | 93% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | MU-MIMO | Zeroforcing | random | 10 | 12.3 | 12 | 92% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | MU-MIMO | cooperative MIMO/precoding | random | 10 | 20.3 | 20 | 91% | Note 1 |
| Source 8 | R1-2110885 | DDDUU | MU-MIMO | Zeroforcing | random | 7 | 6.4 | 6 | 91% | Note 1 |
| Source 8 | R1-2110885 | DDDUU | MU-MIMO | cooperative MIMO/precoding | random | 7 | 12.7 | 12 | 92% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | MU-MIMO | Zeroforcing | random | 7 | 8.4 | 8 | 92% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | MU-MIMO | cooperative MIMO/precoding | random | 7 | 16.9 | 16 | 93% | Note 1 |
| Source 8 | R1-2110885 | DDDUU | MU-MIMO | Zeroforcing | random | 13 | 11.4 | 11 | 92% | Note 1 |
| Source 8 | R1-2110885 | DDDUU | MU-MIMO | cooperative MIMO/precoding | random | 13 | 18.6 | 18 | 92% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | MU-MIMO | Zeroforcing | random | 13 | 14.7 | 14 | 91% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | MU-MIMO | cooperative MIMO/precoding | random | 13 | 22.1 | 22 | 90% | Note 1 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 13.59 | 13 | 92.43% | Note 1 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 14.4 | 14 | 91.84% | Note 1, 7 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 20.78 | 20 | 92.54% | Note 1, 8 |
| Source 6 | R1-2109307 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 7 | 7 | 94.56% | Note 1, 7 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding |  | 10 | 12.5 | 12 | 90% | Note 1, 9 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding |  | 10 | 13.6 | 13 | 92% | Note 1, 9, 10 |
| Source 4 | R1-2111360 | DDDSU | MU-MIMO | reciprocity-based precoding | same | 10 | 5.78 | 5 | 94% | Note 2 |
| Source 10 | R1-2111521 | DDDSU | MU-MIMO |  | random | 10 | 7.15 | 7 | 91.7 | Note 1, 20, 22 |
| Source 10 | R1-2111521 | DDDSU | MU-MIMO |  | random | 10 | 7.5 | 7 | 95.71 | Note 1, 6, 20, 22 |
| Source 10 | R1-2111521 | DDDSU | MU-MIMO |  | random | 10 | 10.57 | 10 | 94.71 | Note 1, 22 |
| Source 10 | R1-2111521 | DDDSU | MU-MIMO |  | random | 10 | 7.59 | 7 | 93.81 | Note 1,20, 21 |
| Source 10 | R1-2111521 | DDDSU | MU-MIMO |  | random | 10 | 10.99 | 10 | 96.09 | Note 1, 21 |
| Source 11 | R1-2111830 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 3.9 | 3 | 99% | Note 2 |
| Source 7 | R1-2112551 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 11.2 | 11 |  | Note 1 |
| Source 7 | R1-2112551 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 12.9 | 12 |  | Note 1, 11 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 13.4 | 13 | 92% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO |  | random | 10 |  | 11 | 95% | Note 1, 12 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO |  | random | 15 |  | 15 | 91% | Note 1, 13 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO |  | random | 20 |  | 16 | 92% | Note 1, 14 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO |  | random | 50 |  | 17 | 94% | Note 1, 15 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO |  | random | 10 |  | 13 | 95% | Note 1, 16 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO |  | random | 15 |  | 16 | 92% | Note 1, 17 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO |  | random | 20 |  | 16 | 95% | Note 1, 18 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO |  | random | 50 |  | 18 | 90% | Note 1, 19 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: BS antenna parameters: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1:8,2) Note 3: DL scheduler for dynamic grant based PDSCH scheduling: Frame Level Integrated Transmission (FLIT) Note 4: X = 99.5 Note 5: X =95 Note 6: Without jitter Note 7: DL scheduler for dynamic grant based PDSCH scheduling: Delay aware (DA) Note 8: stream packet generation rate (Fps or Hz): 120 Note 9: 64QAM Note 10: the traffic model for [3, 109, 91]% relationship Note 11: ADU dropping Note 12: ADU awareness, PDB=10ms: ADU capacity Note 13: ADU awareness, PDB=15ms: ADU capacity Note 14: ADU awareness, PDB=20ms: ADU capacity Note 15: ADU awareness, PDB=50ms: ADU capacity Note 16: ADU awareness, PDB=10ms: PKT capacity Note 17: ADU awareness, PDB=15ms: PKT capacity Note 18: ADU awareness, PDB=20ms: PKT capacity Note 19: ADU awareness, PDB=50ms: PKT capacity Note 20: Target BLER: 1% Note 21: Discard packet not meeting PDB Note 22: Not discard packet not meeting PDB |  |  |  |  |  |  |  |  |  |  |

Table B.1.1.1.1-3. FR1, DL, DU, VR/AR 45Mbps, 60FPS, SU-MIMO, 100Mbps bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 9 | R1-2110811 | DDDSU | SU-MIMO | Close loop rank adaptation | random | 10 | 2.1 | 2 | 91.29% | Note 1 |
| Source 9 | R1-2110811 | DDDSU | SU-MIMO | Close loop rank adaptation | random | 10 | 2.7 | 2 | 95.00% | Note 1, 3 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 5.77 | 5 | 96.51% | Note 1 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 8.03 | 8 | 90.48% | Note 1, 4 |
| Source 4 | R1-2111360 | DDDSU | SU-MIMO | reciprocity-based precoding | same | 10 | 2.04 | 2 | 90% | Note 2 |
| Source 19 | R1-2112573 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 5 | 5 | 94.71% | Note 2 |
| Source 15 | R1-2112572 | DDDSU | SU-MIMO |  | random | 10 | 4.1 | 4 | 92% | Note 2 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 5.2 | 5 | 93% | Note 1, 9 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | reciprocity-based precoding | All Sync | 10 | 4.5 | 4 | 98% | Note 1,10 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | reciprocity-based precoding | Random | 10 | 5.9 | 5 | 99% | Note 1,10 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | reciprocity-based precoding | Evenly Spaced | 10 | 6.1 | 6 | 92% | Note 1,10 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | reciprocity-based precoding | All Sync | 10 | 1.8 | 1 | 97% | Note 1,5,10 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | reciprocity-based precoding | Random | 10 | 3.6 | 3 | 95% | Note 1,5,10 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | reciprocity-based precoding | Evenly Spaced | 10 | 9 | 5 | 90% | Note 1,5,10 |
| Source 5 | R1- 2112079 | DDDSU | SU-MIMO | reciprocity-based precoding | Random | 10 | 1.7 | 1 | 100% | Note 1 |
| Source 7 | R1-2110144 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 5.3 |  |  | Note 1 |
| Source 14 | R1-2112296 | DDDSU | SU-MIMO | codebook-based Type 2 | random | 10 | 6 | 6 | 91.75% | Note 1 |
| Source 14 | R1-2112296 | DDDDD DDDUU (2.6GHz) | SU-MIMO | codebook-based Type 2 | random | 10 | 0 | 0 | N/A | Note 1 |
| Source 14 | R1-2112296 | DSUDD SUUDD (4.9GHz) | SU-MIMO | codebook-based Type 2 | random | 10 | 4.2 | 4 | 91.93% | Note 1 |
| Source 14 | R1-2112296 | DDDDD DDDUU (2.6GHz) + DSUDD SUUDD (4.9GHz) | SU-MIMO | codebook-based Type 2 | random | 10 | 10.3 | 10 | 91.53% | Note 1 |
| Source 14 | R1-2112296 | DDDDD DDDUU (2.6GHz) + DSUDD SUUDD (4.9GHz) | SU-MIMO | codebook-based Type 2 | random | 10 | 12.3 | 12 | 92.15% | Note 1 |
| Source 17 | R1-2111349 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 5.2 | 5 | 94% | Note 2 |
| Source 17 | R1-2111349 | DDDSU | SU-MIMO | reciprocity-based precoding | evenly spaced | 10 | 5.4 | 5 | 97% | Note 2 |
| Source 17 | R1-2111349 | DDDSU | SU-MIMO | reciprocity-based precoding | same | 10 | 4.4 | 4 | 96% | Note 2 |
| Source 17 | R1-2111349 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 5.4 | 5 | 95% | Note 2, 8 |
| Source 17 | R1-2111349 | DDDSU | SU-MIMO | reciprocity-based precoding | evenly spaced | 10 | 6.6 | 6 | 96.49 | Note 2,8 |
| Source 17 | R1-2111349 | DDDSU | SU-MIMO | reciprocity-based precoding | same | 10 | 4.4 | 4 | 97% | Note 2,8 |
| Source 8 | R1-2108799 | DDDSU | SU-MIMO | Zeroforcing | random | 10 | 6 | 6 | 90% | Note 1 |
| Source 8 | R1-2108799 | DDDSU | SU-MIMO | cooperative MIMO/precoding | random | 10 | 7 | 7 | 90% | Note 1 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: BS antenna parameters: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1:8,2) Note 3: DL scheduler for dynamic grant based PDSCH scheduling: Frame Level Integrated Transmission (FLIT) Note 4: stream packet generation rate (Fps or Hz): 120 Note 5: stream packet generation rate (Fps or Hz): 30 Note 6: 64QAM Note 7: Jitter STD=2ms, Jitter range Min=0ms, Jitter range Max=8ms Note 8: Without jitter Note 9: Discard packet not meeting PDB Not 10: Not discard packet not meeting PDB |  |  |  |  |  |  |  |  |  |  |

Table B.1.1.1.1-4. FR1, DL, DU, VR/AR 45Mbps, 60FPS, MU-MIMO, 100Mbps bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | Close loop rank adaptation | random | 10 | 5.3 | 5 | 91.90% | Note 1 |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | Close loop rank adaptation | random | 10 | 6.6 | 6 | 92.59% | Note 1, 3 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 6.91 | 6 | 95.63% | Note 1 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 11.42 | 11 | 91.77% | Note 1, 4 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding |  | 10 | 7.8 | 7 | 97% | Note 1, 5 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding |  | 10 | 7.9 | 7 | 97% | Note 1, 5, 6 |
| Source 7 | R1-2110403 | DDDSU | MU-MIMO |  | random | 10 | 6.4 |  |  | Note 1 |
| Source 11 | R1-2111830 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 2.4 | 2 | 95% | Note 2 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 8.4 | 8 | 92% | Note 1 |
| Source 8 | R1-2108799 | DDDSU | SU-MIMO | Zeroforcing | random | 10 | 7.6 | 7 | 91% | Note 2 |
| Source 8 | R1-2108799 | DDDSU | SU-MIMO | cooperative MIMO/precoding | random | 10 | 14.3 | 14 | 91% | Note 2 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: BS antenna parameters: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1:8,2) Note 3: DL scheduler for dynamic grant based PDSCH scheduling: Frame Level Integrated Transmission (FLIT) Note 4: stream packet generation rate (Fps or Hz): 120 Note 5: 64QAM Note 6: the traffic model for [3, 109, 91]% relationship |  |  |  |  |  |  |  |  |  |  |

Table B.1.1.1.1-5. FR1, DL, DU, VR/AR 60Mbps, 60FPS, MU-MIMO, 100Mbps bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 4.6 | 4 | 94.50% | Note 1,2 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 0 | 0 | N.A. | Note 1,3 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 2.8 | 2 | 92.90% | Note 1,4 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 0 | 0 | N.A. | Note 1,5 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 2 | 2 | 90.10% | Note 1,6 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 0 | 0 | N.A. | Note 1,7 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: Soft HARQ-Ack, k3 = 4 Note 3: Baseline HARQ-Ack, k3 = 4 Note 4: Soft HARQ-Ack, k3 = 6 Note 5: Baseline HARQ-Ack, k3 = 6 Note 6: Soft HARQ-Ack, k3 = 8 Note 7: Baseline HARQ-Ack, k3 = 8 |  |  |  |  |  |  |  |  |  |  |

        - Multi-stream traffic model

Table B.1.1.1.2-1. FR1, DL, DU, GOP-based 30Mbps, SU-MIMO, 100Mbps bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Traffic arrival offset among different UEs | α | [I_PDB, P_PDB] (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 14 | R1-2109555 | DDDSU | SU-MIMO | random | 2 | [10,10] | 6 | 6 | 93.34% | Note 1,2 |
| Source 14 | R1-2109555 | DDDSU | SU-MIMO | random | 2 | [10,10] | 6 | 6 | 93.81% | Note 1,3 |
| Source 14 | R1-2109555 | DDDSU | SU-MIMO | random | 2 | [10,10] | 2 | 2 | 91.91% | Note 1,4 |
| Source 14 | R1-2109555 | DDDSU | SU-MIMO | random | 2 | [17, 9] | 9 | 9 | 89.60% | Note 1,2 |
| Source 14 | R1-2109555 | DDDSU | SU-MIMO | random | 2 | [15,10] | 10 | 10 | 90.39% | Note 1,2 |
| Source 14 | R1-2109555 | DDDSU | SU-MIMO | random | 2 | [15,10] | 10 | 10 | 94.00% | Note 1,3 |
| Source 14 | R1-2109555 | DDDSU | SU-MIMO | random | 2 | [10,10] | 8 | 8 | 94.05% | Note 1,2,5 |
| Source 14 | R1-2109555 | DDDSU | SU-MIMO | random | 2 | [10,10] | 8 | 8 | 94.41% | Note 1,3,5 |
| Source 14 | R1-2109555 | DDDSU | SU-MIMO | random | 2 | [10,10] | 2 | 2 | 89.53% | Note 1,4,5 |
| Source 14 | R1-2109555 | DDDSU | SU-MIMO | random | 2 | [17, 9] | 11 | 11 | 88.30% | Note 1,2,5 |
| Source 14 | R1-2109555 | DDDSU | SU-MIMO | random | 2 | [15,10] | 11 | 11 | 90.65% | Note 1,2,5 |
| Source 14 | R1-2109555 | DDDSU | SU-MIMO | random | 2 | [10,10] | 6 | 6 | 93.34% | Note 1,3,5 |
| Source 5 | R1-2112079 | DDDSU | SU-MIMO | random | 1..5 | [10,10] | 6.5 | 6 | 98.40% | Note 1, 2 |
| Source 5 | R1-2112079 | DDDSU | SU-MIMO | random | 2 | [10,10] | 6.1 | 6 | 92.11% | Note 1, 2 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: [PER_I, PER_P] = [1%, 1%] Note 3: [PER_I, PER_P] = [1%, 5%] Note 4: [PER_I, PER_P] = [0.5%, 5%] Note 5: Delay aware scheduler |  |  |  |  |  |  |  |  |  |  |

Table B.1.1.1.2-2. FR1, DL, DU, GOP-based 30Mbps, MU-MIMO, 100Mbps bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Traffic arrival offset among different UEs | α | [I_PDB, P_PDB] (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | random | 1 | [10,10] | 10 | 10 | 90.08% | Note 1,2 |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | random | 2 | [10,10] | 6.7 | 6 | 93.12% | Note 1,2 |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | random | 2 | [15, 9] | 8.8 | 8 | 94.35% | Note 1,2 |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | random | 2 | [10,10] | 6.7 | 6 | 93.12% | Note 1,3 |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | random | 2 | [15,10] | 9.1 | 9 | 90.87% | Note 1,2 |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | random | 2 | [15,10] | 9.6 | 9 | 92.06% | Note 1,3 |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | random | 2 | [10,10] | 6 | 6 | 90.08% | Note 1,4 |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | random | 2 | [17, 9] | 9.5 | 9 | 91.45% | Note 1,2 |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | random | 2 | [17, 10] | 10.5 | 10 | 91.59% | Note 1,2 |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | random | 2 | [17, 10] | 11.8 | 11 | 93.51% | Note 1,3 |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | random | 2 | [10,10] | 7.4 | 7 | 91.38% | Note 1,4,5 |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | random | 2 | [10,10] | 8.6 | 8 | 95.44% | Note 1,4,6 |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | random | 1.5 | [10,10] | 8.5 | 8 | 93.95% | Note 1,2 |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | random | 3 | [10,10] | 4 | 4 | 90.12% | Note 1,2 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 1.5 | [10,10] | 6.74 | 6 | 93.12% | Note 1,2 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 1.5 | [10,10] | 6.74 | 6 | 93.12% | Note 1,3 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 1.5 | [10,10] | 6.39 | 6 | 91.67% | Note 1,4 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 1.5 | [15,10] | 12.58 | 12 | 92.20% | Note 1,2 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 1.5 | [15,10] | 12.8 | 12 | 92.86% | Note 1,3 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 1.5 | [15,10] | 12.25 | 12 | 91.14% | Note 1,4 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 1.5 | [15,9] | 12.39 | 12 | 91.53% | Note 1,2 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 1.5 | [15,9] | 12.53 | 12 | 92.06% | Note 1,3 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 1.5 | [15,9] | 12.2 | 12 | 90.87% | Note 1,4 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 2 | [10,10] | 5.2 | 5 | 91.14% | Note 1,2 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 2 | [10,10] | 5.2 | 5 | 91.14% | Note 1,3 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 2 | [10,10] | 4.74 | 4 | 94.84% | Note 1,4 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 2 | [10,10] | 5.35 | 5 | 91.47% | Note 1,2,5 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 2 | [10,10] | 5.35 | 5 | 91.47% | Note 1,3,5 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 2 | [10,10] | 4.97 | 4 | 90.87% | Note 1,4,5 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 2 | [15,10] | 10.06 | 10 | 90.32% | Note 1,2 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 2 | [15,10] | 10.06 | 10 | 90.32% | Note 1,3 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 2 | [15,10] | 9.12 | 9 | 90.40% | Note 1,4 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 2 | [15,9] | 9.19 | 9 | 92.70% | Note 1,2 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 2 | [15,9] | 9.97 | 9 | 92.83% | Note 1,3 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 2 | [15,9] | 8.99 | 8 | 93.55% | Note 1,4 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 3 | [10,10] | 2.21 | 2 | 92.86% | Note 1,2 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 3 | [10,10] | 2.21 | 2 | 92.86% | Note 1,3 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 3 | [10,10] | 2.09 | 2 | 91.27% | Note 1,4 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 3 | [15,10] | 5.73 | 5 | 93.58% | Note 1,2 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 3 | [15,10] | 5.73 | 5 | 93.75% | Note 1,3 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 3 | [15,10] | 4.91 | 4 | 94.44% | Note 1,4 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 3 | [15,9] | 5.69 | 5 | 93.17% | Note 1,2 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 3 | [15,9] | 5.69 | 5 | 93.17% | Note 1,3 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 3 | [15,9] | 4.84 | 4 | 93.58% | Note 1,4 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO |  | 2 | 10 | 10.8 | 10 | 94% | Note 1, 2, 9 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO |  | 2 | 10 | 12.2 | 12 | 92% | Note 1, 7, 9 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO |  | 2 | 10 | 10.9 | 10 | 94% | Note 1, 8, 9 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO |  | 2 | 10 | 10.9 | 10 | 94% | Note 1, 3, 9 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: [PER_I, PER_P] = [1%, 1%] Note 3: [PER_I, PER_P] = [1%, 5%] Note 4: [PER_I, PER_P] = [0.5%, 5%] Note 5: Based on PF, prioritize the transmission of I frame Note 6: [PER_I, PER_P] = FLIT and prioritize the transmission of I frame Note 7: [PER_I, PER_P] = [10%, 1%] Note 8: [PER_I, PER_P] = [1%, 10%] Note 9: 64QAM |  |  |  |  |  |  |  |  |  |  |

Table B.1.1.1.2-3. FR1, DL, DU, GOP-based 45Mbps, SU-MIMO, 100Mbps bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Traffic arrival offset among different UEs | α | [I_PDB, P_PDB] (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 14 | R1-2109555 | DDDSU | SU-MIMO | random | 1.5 | [10,10] | 2 | 2 | 89.05% | Note 1,2 |
| Source 14 | R1-2109555 | DDDSU | SU-MIMO | random | 1.5 | [10,10] | 3 | 3 | 89.53% | Note 1,2,4 |
| Source 14 | R1-2109555 | DDDSU | SU-MIMO | random | 1.5 | [10,10] | 3 | 3 | 90.16% | Note 1,3,4 |
| Source 14 | R1-2109555 | DDDSU | SU-MIMO | random | 1.5 | [17, 9] | 4 | 4 | 89.77% | Note 1,2,4 |
| Source 14 | R1-2109555 | DDDSU | SU-MIMO | random | 1.5 | [15,10] | 4 | 4 | 88.58% | Note 1,2 |
| Source 14 | R1-2109555 | DDDSU | SU-MIMO | random | 1.5 | [15,10] | 5 | 5 | 91.24% | Note 1,3 |
| Source 14 | R1-2109555 | DDDSU | SU-MIMO | random | 1.5 | [15,10] | 5 | 5 | 89.72% | Note 1,2,4 |
| Source 14 | R1-2109555 | DDDSU | SU-MIMO | random | 1.5 | [15,10] | 6 | 6 | 89.21% | Note 1,3,4 |
| Source 14 | R1-2109555 | DDDSU | SU-MIMO | random | 3 | [10,10] | <2 | <2 | N/A | Note 1,2 |
| Source 14 | R1-2109555 | DDDSU | SU-MIMO | random | 3 | [10,10] | 2 | 2 | 87.62% | Note 1,2,4 |
| Source 14 | R1-2109555 | DDDSU | SU-MIMO | random | 3 | [10,10] | 2 | 2 | 89.53% | Note 1,3,4 |
| Source 14 | R1-2109555 | DDDSU | SU-MIMO | random | 3 | [15,10] | 4 | 4 | 95.00% | Note 1,3 |
| Source 14 | R1-2109555 | DDDSU | SU-MIMO | random | 3 | [15,10] | 4 | 4 | 96.91% | Note 1,2 |
| Source 14 | R1-2109555 | DDDSU | SU-MIMO | random | 3 | [15,10] | 6 | 6 | 88.26% | Note 1,3,4 |
| Source 14 | R1-2109555 | DDDSU | SU-MIMO | random | 3 | [15,10] | 6 | 6 | 89.85% | Note 1,2,4 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: [PER_I, PER_P] = [1%, 1%] Note 3: [PER_I, PER_P] = [1%, 5%] Note 4: Delay aware scheduler |  |  |  |  |  |  |  |  |  |  |

Table B.1.1.1.2-4. FR1, DL, DU, GOP-based 45Mbps, MU-MIMO, 100Mbps bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Traffic arrival offset among different UEs | α | [I_PDB, P_PDB] (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | random | 1.5 | [10,10] | 1.4 | 1 | 97.14% | Note 1,2 |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | random | 1.5 | [10,10] | 2.6 | 2 | 92.83% | Note 1,2,3 |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | random | 1.5 | [10,10] | 3.2 | 3 | 90.79% | Note 1,2,4 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: [PER_I, PER_P] = [0.5%, 5%] Note 3: Based on PF, prioritize the transmission of I frame Note 4: [PER_I, PER_P] = FLIT and prioritize the transmission of I frame |  |  |  |  |  |  |  |  |  |  |

Table B.1.1.1.2-5. FR1, DL, DU, Slice-based 30Mbps, MU-MIMO, 100Mbps bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Traffic arrival offset among different UEs | α | [I_PDB, P_PDB] (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | random | 2 | [10,10] | 14.9 | 14 | 91.67% | Note 1,2 |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | random | 2 | [10,10] | 15.7 | 15 | 91.17% | Note 1,4 |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | random | 2 | [10,10] | 17.3 | 17 | 90.87% | Note 1,3 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 1.5 | [10,10] | 13.78 | 13 | 92.38% | Note 1,2 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 1.5 | [10,10] | 16.74 | 16 | 91.52% | Note 1,3 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 1.5 | [10,10] | 16.74 | 16 | 91.52% | Note 1,4 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 1.5 | [15,10] | 13.93 | 13 | 92.87% | Note 1,2 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 1.5 | [15,10] | 16.79 | 16 | 91.72% | Note 1,3 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 1.5 | [15,10] | 16.77 | 16 | 91.62% | Note 1,4 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 1.5 | [15,9] | 13.27 | 13 | 90.86% | Note 1,2 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 1.5 | [15,9] | 16.37 | 16 | 90.92% | Note 1,3 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 1.5 | [15,9] | 16.33 | 16 | 90.82% | Note 1,4 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 2 | [10,10] | 13.69 | 13 | 92.25% | Note 1,2 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 2 | [10,10] | 16.84 | 16 | 91.77% | Note 1,3 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 2 | [10,10] | 16.59 | 16 | 91.27% | Note 1,4 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 2 | [10,10] | 13.54 | 13 | 91.72% | Note 1,2,5 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 2 | [10,10] | 16.23 | 16 | 90.77% | Note 1,3,5 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 2 | [10,10] | 16.17 | 16 | 90.57% | Note 1,4,5 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 2 | [15,10] | 13.73 | 13 | 92.44% | Note 1,2 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 2 | [15,10] | 16.95 | 16 | 91.96% | Note 1,3 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 2 | [15,10] | 16.8 | 16 | 91.67% | Note 1,4 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 2 | [15,9] | 13.36 | 13 | 91.21% | Note 1,2 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 2 | [15,9] | 16.74 | 16 | 91.46% | Note 1,3 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 2 | [15,9] | 16.66 | 16 | 91.36% | Note 1,4 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 3 | [10,10] | 13.77 | 13 | 92.46% | Note 1,2 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 3 | [10,10] | 16.89 | 16 | 91.67% | Note 1,3 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 3 | [10,10] | 16.89 | 16 | 91.67% | Note 1,4 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 3 | [15,10] | 13.84 | 13 | 92.63% | Note 1,2 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 3 | [15,10] | 16.98 | 16 | 92.06% | Note 1,3 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 3 | [15,10] | 16.89 | 16 | 91.85% | Note 1,4 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 3 | [15,9] | 13.46 | 13 | 91.43% | Note 1,2 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 3 | [15,9] | 16.75 | 16 | 91.54% | Note 1,3 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | random | 3 | [15,9] | 16.72 | 16 | 91.48% | Note 1,4 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO |  |  | [10,10] | 12.7 | 12 | 93% | Note 1, 2, 6 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO |  |  | [10,10] | 14.6 | 14 | 91% | Note 1, 3, 6 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: [PER_I, PER_P] = [1%, 1%] Note 3: [PER_I, PER_P] = [1%, 5%] Note 4: [PER_I, PER_P] = [0.5%, 5%] Note 5: Based on PF, prioritize the transmission of I frame Note 6: 64QAM |  |  |  |  |  |  |  |  |  |  |

Table B.1.1.1.2-6. FR1, DL, DU, Video stream 30Mbps+Data/audio stream 1.12Mbps + pose/control 0.2 Mbps , SU-MIMO, 100Mbps bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme Traffic arrival offset among different UEs | [Video_PDB, Data/audio_PDB] (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 1 | R1-2111902 | DDDSU | SU-MIMO |  | [10, 30] | 6 | 6 |  | Note 1 |
| Note 1: BS antenna parameters: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1,8,2) |  |  |  |  |  |  |  |  |  |

      - CG

Table B.1.1.2-1. FR1, DL, DU, CG 8Mbps, 60FPS, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 15 | 24.4 | 24 | 93% | Note 1 |
| Source 5 | R1- 2112079 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | >30 | >30 | N/A | Note 1 |
| Source 14 | R1-2112296 | DDDSU | SU-MIMO | codebook-based Type 2 | random | 15 | >20 | >20 | N/A | Note 1 |
| Source 7 | R1-2110144 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 15 | >36 |  |  | Note 1 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |  |  |  |  |  |  |  |  |  |  |

Table B.1.1.2-2. FR1, DL, DU, CG 8Mbps, 60FPS, MU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 15 | 56.6 | 56 | 92% | Note 1 |
| Source 7 | R1-2110144 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 15 | >36 |  |  | Note 1 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |  |  |  |  |  |  |  |  |  |  |

Table B.1.1.2-3. FR1, DL, DU, CG 30Mbps, 60FPS, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 9 | R1-2110811 | DDDSU | SU-MIMO | Close loop rank adaptation | random | 15 | 7.6 | 7 | 92.52% | Note 1 |
| Source 8 | R1-2110885 | DDDUU | SU-MIMO | Zeroforcing | random | 15 | 10.3 | 10 | 91% | Note 1 |
| Source 8 | R1-2110885 | DDDUU | SU-MIMO | cooperative MIMO/precoding | random | 15 | 11.4 | 11 | 92% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | SU-MIMO | Zeroforcing | random | 15 | 12.4 | 12 | 91% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | SU-MIMO | cooperative MIMO/precoding | random | 15 | 14.9 | 14 | 92% | Note 1 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 15 | 11.68 | 11 | 94.81% | Note 1 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 15 | 13.58 | 13 | 94.90% | Note 1, 3 |
| Source 3 | R1-2111234 | DDDSU | SU-MIMO | codebook-based Type 2 | random | 15 | 10 | 10 | 92% | Note 4, 5 |
| Source 4 | R1-2108869 | DDDSU | SU-MIMO | reciprocity-based precoding | same | 15 | 5.57 | 5 | 94% | Note 2 |
| Source 10 | R1-2111521 | DDDSU | SU-MIMO |  | random | 15 | 6.17 | 6 | 91.01% | Note 1 |
| Source 10 | R1-2111521 | DDDSU | SU-MIMO |  | random | 15 | 7.99 | 7 | 97.14% | Note 1, 8 |
| Source 19 | R1-2112573 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 15 | 8 | 8 | 92.88% | Note 2 |
| Source 6 | R1-2111632 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 15 | 10 | 10 | 91.46% | Note 1, 3 |
| Source 15 | R1-2112572 | DDDSU | SU-MIMO |  | random | 15 | 8.5 | 8 | 97% | Note 2 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 15 | 10 | 10 | 91% | Note 1 |
| Source 5 | R1- 2112079 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 15 | 6.7 | 6 | 100% | Note 1 |
| Source 14 | R1-2112296 | DDDSU | SU-MIMO | codebook-based Type 2 | random | 15 | 13 | 13 | 90.41% | Note 1 |
| Source 17 | R1-2111349 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 15 | 10.2 | 10 | 92% | Note 2 |
| Source 17 | R1-2111349 | DDDSU | SU-MIMO | reciprocity-based precoding | evenly spaced | 15 | 10.3 | 10 | 93% | Note 2 |
| Source 17 | R1-2111349 | DDDSU | SU-MIMO | reciprocity-based precoding | same | 15 | 10.3 | 10 | 94% | Note 2 |
| Source 17 | R1-2111349 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 15 | 10.5 | 10 | 94% | Note 2, 6 |
| Source 17 | R1-2111349 | DDDSU | SU-MIMO | reciprocity-based precoding | evenly spaced | 15 | 11 | 11 | 91% | Note 2, 6 |
| Source 17 | R1-2111349 | DDDSU | SU-MIMO | reciprocity-based precoding | same | 15 | 10.1 | 10 | 93% | Note 2, 6 |
| Source 7 | R1-2110144 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 15 | 11 |  |  | Note 1 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: BS antenna parameters: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1:8,2) Note 3: DL scheduler for dynamic grant based PDSCH scheduling: Delay aware (DA) Note 4: 64QAM Note 5: Jitter STD=2ms, Jitter range Min=0ms, Jitter range Max=8ms Note 6: Without jitter Note 7: the traffic model for [3, 109, 91]% relationship Note 8: Target BLER = 1% |  |  |  |  |  |  |  |  |  |  |

Table B.1.1.2-4. FR1, DL, DU, CG 30Mbps, 60FPS, MU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | Close loop rank adaptation | random | 15 | 16.1 | 16 | 90.77% | Note 1 |
| Source 8 | R1-2110885 | DDDUU | MU-MIMO | Zeroforcing | random | 15 | 12.3 | 12 | 92% | Note 1 |
| Source 8 | R1-2110885 | DDDUU | MU-MIMO | cooperative MIMO/precoding | random | 15 | 19.7 | 19 | 92% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | MU-MIMO | Zeroforcing | random | 15 | 17.1 | 17 | 91% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | MU-MIMO | cooperative MIMO/precoding | random | 15 | 22.9 | 22 | 91% | Note1 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 15 | 19.65 | 19 | 92.56% | Note 1 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 15 | 19.75 | 19 | 92.86% | Note 1, 3 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding |  | 15 | 14.7 | 14 | 93% | Note 1, 4 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding |  | 15 | 14.8 | 14 | 93% | Note 1, 4, 5 |
| Source 4 | R1-2111360 | DDDSU | MU-MIMO | reciprocity-based precoding | same | 15 | >8 | 8 | 91% | Note 2 |
| Source 10 | R1-2111521 | DDDSU | MU-MIMO |  | random | 15 | 7.47 | 7 | 94.35 | Note 1, 6 |
| Source 10 | R1-2111521 | DDDSU | MU-MIMO |  | random | 15 | 8.2 | 8 | 90.14 | Note 1, 6,7 |
| Source 10 | R1-2111521 | DDDSU | MU-MIMO |  | random | 15 | 11.26 | 11 | 91.82 | Note 1 |
| Source 6 | R1-2111632 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 15 | 10.1 | 10 | 90.53% | Note 1, 3 |
| Source 11 | R1-2111830 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 15 | 5 | 5 | 90% | Note 2 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 15 | 16.5 | 16 | 93% | Note 1 |
| Source 7 | R1-2110144 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 15 | 15.1 |  |  | Note 1 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: BS antenna parameters: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1:8,2) Note 3: DL scheduler for dynamic grant based PDSCH scheduling: Delay aware (DA) Note 4: 64QAM Note 5: the traffic model for [3, 109, 91]% relationship Note 6: Target BLER = 1% Note 7: Without jitter |  |  |  |  |  |  |  |  |  |  |

Table B.1.1.2-5. FR1, DL, DU, CG 45Mbps, 60FPS, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 17 | R1-2111349 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 15 | 6.3 | 6 | 94% | Note 1 |
| Source 17 | R1-2111349 | DDDSU | SU-MIMO | reciprocity-based precoding | evenly spaced | 15 | 6.3 | 6 | 94% | Note 1 |
| Source 17 | R1-2111349 | DDDSU | SU-MIMO | reciprocity-based precoding | same | 15 | 6.4 | 6 | 96% | Note 1 |
| Source 17 | R1-2111349 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 15 | 6.7 | 6 | 98% | Note 1, 2 |
| Source 17 | R1-2111349 | DDDSU | SU-MIMO | reciprocity-based precoding | evenly spaced | 15 | 7.1 | 7 | 90% | Note 1, 2 |
| Source 17 | R1-2111349 | DDDSU | SU-MIMO | reciprocity-based precoding | same | 15 | 6.3 | 6 | 95% | Note 1, 2 |
| Note 1: BS antenna parameters: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1:8,2) Note 2: Without jitter |  |  |  |  |  |  |  |  |  |  |

    - InH Scenario

      - VR/AR

        - Single stream traffic model

Table B.1.2.1.1-1. FR1, DL, InH, VR/AR 30Mbps, 60FPS, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 8.27 | 8 | 92.71% |  |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 10.77 | 10 | 95.20% | Note 1 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 11.63 | 11 | 95.28% | Note 2 |
| Source 19 | R1-2112573 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 7 | 7 | 91.82% |  |
| Source 15 | R1-2112572 | DDDSU | SU-MIMO |  | random | 10 | 5.2 | 5 | 94% |  |
| Source 12 | R1-2112175 | DDDSU | SU-MIMO |  | synchronized | 10 | 4.85 | 4 | 100.00% |  |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 7 | 7 | 91% |  |
| Source 14 | R1-2112296 | DDDSU | SU-MIMO | codebook-based Type 2 | random | 10 | 8 | 8 | 88.13% |  |
| Source 7 | R1-2110144 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 8.5 |  |  |  |
| Source 6 | R1-2109307 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 1 | 1 | 100% | Note 1 |
| Note 1: DL scheduler for dynamic grant based PDSCH scheduling: Delay aware (DA) Note 2: stream packet generation rate (Fps or Hz): 120 |  |  |  |  |  |  |  |  |  |  |

Table B.1.2.1.1-2. FR1, DL, InH, VR/AR 30Mbps, 60FPS, MU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 10.8 | 10 | 92.50% |  |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 12.4 | 12 | 93.06% | Note 1 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 16.53 | 16 | 92.71% | Note 2 |
| Source 3 | R1-2111234 | DDDSU | MU-MIMO | codebook-based Type 2 | random | 10 | 12 | 12 | 96% |  |
| Source 3 | R1-2111234 | DDDSU | MU-MIMO | codebook-based Type 2 | random | 7 | 8 | 8 | 96% |  |
| Source 3 | R1-2111234 | DDDSU | MU-MIMO | codebook-based Type 2 | random | 10 | 16 | 16 | 95% | Note 3, 4,5 |
| Source 3 | R1-2111234 | DDDSU | MU-MIMO | codebook-based Type 2 | random | 10 | 20 | 20 | 92% | Note 3, 4,6 |
| Source 3 | R1-2111234 | DDDSU | MU-MIMO | codebook-based Type 2 | random | 10 | 20 | 20 | 91% | Note 3, 4,7 |
| Source 3 | R1-2111234 | DDDSU | MU-MIMO | codebook-based Type 2 | random | 10 | 12 | 12 | 90% | Note 3, 4,8 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding |  | 10 | 11.4 | 11 | 92% | Note 3 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding |  | 10 | 11.8 | 11 | 94% | Note 9 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding |  | 10 | 16.6 | 16 | 91% | Note 3, 10 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding |  | 10 | 11.8 | 11 | 94% | Note 3, 11 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding |  | 10 | 8.5 | 8 | 95% | Note 3, 12 |
| Source 11 | R1-2111830 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 5.8 | 5 | 96.80% |  |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 10.3 | 10 | 93% |  |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO |  | random | 10 |  | 9 | 91% | Note 13 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO |  | random | 15 |  | 11 | 92% | Note 14 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO |  | random | 20 |  | 12 | 93% | Note 15 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO |  | random | 50 |  | 13 | 94% | Note 16 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO |  | random | 10 |  | 10 | 94% | Note 17 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO |  | random | 15 |  | 12 | 93% | Note 18 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO |  | random | 20 |  | 12 | 95% | Note 19 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO |  | random | 50 |  | 13 | 95% | Note 20 |
| Source 7 | R1-2110144 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 9.2 |  |  |  |
| Source 6 | R1-2109307 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 5 | 5 | 91.67% | Note 1 |
| Note 1: DL scheduler for dynamic grant based PDSCH scheduling: Delay aware (DA) Note 2: stream packet generation rate (Fps or Hz): 120 Note 3: 64QAM Note 4: Jitter STD=2ms, Jitter range Min=0ms, Jitter range Max=8ms Note 5: gNB scheduling awareness of 2 frames UE playout buffer Note 6: gNB scheduling awareness of 3 frames UE playout buffer Note 7: gNB scheduling awareness of 4 frames UE playout buffer Note 8: XR-dedicated PDCCH monitoring window Note 9: the traffic model for [3, 109, 91]% relationship Note 10: Ehanced Preemption (XR vs. uRLLC) Note 11: Rel-15 Preemption (XR vs. uRLLC) Note 12: No Preemption (XR vs. uRLLC) Note 13: ADU awareness, PDB=10ms: ADU capacity Note 14: ADU awareness, PDB=15ms: ADU capacity Note 15: ADU awareness, PDB=20ms: ADU capacity Note 16: ADU awareness, PDB=50ms: ADU capacity Note 17: ADU awareness, PDB=10ms: PKT capacity Note 18: ADU awareness, PDB=15ms: PKT capacity Note 19: ADU awareness, PDB=20ms: PKT capacity Note 20: ADU awareness, PDB=50ms: PKT capacity |  |  |  |  |  |  |  |  |  |  |

Table B.1.2.1.1-3. FR1, DL, InH, VR/AR 45Mbps, 60FPS, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 4.65 | 4 | 97.22% |  |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 6.59 | 6 | 97.22% | Note 1 |
| Source 19 | R1-2112573 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 5 | 5 | 93.25% |  |
| Source 15 | R1-2112572 | DDDSU | SU-MIMO |  | random | 10 | 3.27 | 3 | 97% |  |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 4.3 | 4 | 97% |  |
| Source 14 | R1-2112296 | DDDSU | SU-MIMO | codebook-based Type 2 | random | 10 | 4.6 | 4 | 96.30% |  |
| Source 7 | R1-2110144 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 4.8 |  |  |  |
| Note 1: stream packet generation rate (Fps or Hz): 120 |  |  |  |  |  |  |  |  |  |  |

Table B.1.2.1.1-4. FR1, DL, InH, VR/AR 45Mbps, 60FPS, MU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 5.91 | 5 | 96.67% |  |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 9.22 | 9 | 91.36% | Note 1 |
| Source 3 | R1-2109200/R1-2111234 | DDDSU | MU-MIMO | codebook-based Type 2 | random | 10 | 8 | 8 | 94% | Note 2, 3 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding |  | 10 | 7.2 | 7 | 92% | Note 2 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding |  | 10 | 7.3 | 7 | 93% | Note 2, 4 |
| Source 11 | R1-2111830 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 3.5 | 3 | 98% |  |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 6.4 | 6 | 93% |  |
| Source 7 | R1-2110144 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 5.4 |  |  |  |
| Note 1: stream packet generation rate (Fps or Hz): 120 Note 2: 64QAM Note 3: Jitter STD=2ms, Jitter range Min=0ms, Jitter range Max=8ms Note 4: the traffic model for [3, 109, 91]% relationship |  |  |  |  |  |  |  |  |  |  |

Table B.1.2.1.1-5. FR1, DL, InH, VR/AR 60Mbps, 60FPS, MU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 3 | R1-2111234 | DDDSU | MU-MIMO | codebook-based Type 2 | random | 10 | 4 | 4 | 100% | Note 1, 2 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 2.93 | 2 | 97.70% | Note 3 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 0 | 0 | N.A. | Note 4 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 2.1 | 2 | 91.25% | Note 5 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 0 | 0 | N.A. | Note 6 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 1.17 | 1 | 91.25% | Note 7 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 0 | 0 | N.A. | Note 8 |
| Note 1: 64QAM Note 2: Jitter STD=2ms, Jitter range Min=0ms, Jitter range Max=8ms Note3: Soft HARQ-Ack, k3 = 4 Note4: Baseline HARQ-Ack, k3 = 4 Note5: Soft HARQ-Ack, k3 = 6 Note6: Baseline HARQ-Ack, k3 = 6 Note7: Soft HARQ-Ack, k3 = 8 Note8: Baseline HARQ-Ack, k3 = 8 |  |  |  |  |  |  |  |  |  |  |

        - Multi-stream traffic model

Table B.1.2.1.2-1. FR1, DL, InH, Video stream 30Mbps+Data/audio stream 1.12Mbps, SU-MIMO, 100MHz bandwidth

| Source 1Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | [Video_PDB, Data/audio_PDB] (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 1 | R1-2111902 | DDDSU | SU-MIMO |  | Random | [10,30] | 4.1 | 4 | 91% |  |
| Source 20 | R1-2111531 | DDDSU | MU-MIMO | reciprocity-based precoding | Random | [10,10] | 8.4 | 8 | 92% | Note 1, 2 |
| Source 20 | R1-2111531 | DDDSU | MU-MIMO | reciprocity-based precoding | Random | [10,10] | 5.7 | 5 | 95% | Note 1, 3 |
| Source 20 | R1-2111531 | DDDSU | MU-MIMO | reciprocity-based precoding | Random | [10,10] | 4.9 | 4 | 92% | Note 1, 4 |
| Note 1: 64QAM Note 2: Enhanced Preemption (Audio/data streams vs. Video streams) Note 3: Rel-15 Preemption(Audio/data streams vs. Video streams) Note 4: No Preemption (Audio/data streams vs. Video streams) |  |  |  |  |  |  |  |  |  |  |

Table B.1.2.1.2-2. FR1, DL, InH, slice-based multi-streams traffic model and single stream video traffic model, MU-MIMO

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | [I_PDB, P_PDB, PDB_video] (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding | Random | [10,10, 10] | 10.2 | 10 | 90% | Note 1, 2 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding | Random | [10,10, 10] | 7.1 | 7 | 90% | Note 1, 3 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding | Random | [10,10, 10] | 4.5 | 4 | 93% | Note 1, 4 |

      - CG

Table B.1.2.2-1. FR1, DL, InH, CG 8Mbps, 60FPS, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 15 | 22.3 | 22 | 94% |  |
| Source 14 | R1-2112296 | DDDSU | SU-MIMO | codebook-based Type 2 | random | 15 | >20 | >20 | N/A |  |
| Source 7 | R1-2110144 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 15 | >38.7 |  |  |  |

Table B.1.2.2-2. FR1, DL, InH, CG 8Mbps, 60FPS, MU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 15 | 44.1 | 44 | 90% |  |
| Source 7 | R1-2110144 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 15 | >38.7 |  |  |  |

Table B.1.2.2-3. FR1, DL, InH, CG 30Mbps, 60FPS, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 15 | 10.14 | 10 | 91.67% |  |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 15 | 11.43 | 11 | 96.06% | Note 1 |
| Source 19 | R1-2112573 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 15 | 8 | 8 | 93.54% |  |
| Source 6 | R1-2111632 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 15 | 6.8 | 6 | 92.98% | Note 1 |
| Source 15 | R1-2112572 | DDDSU | SU-MIMO |  | random | 15 | 5.96 | 5 | 99% |  |
| Source 12 | R1-2112175 | DDDSU | SU-MIMO |  | synchronized | 15 | 9.4 | 9 | 91.67% |  |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 15 | 8.4 | 8 | 97.5 |  |
| Source 14 | R1-2112296 | DDDSU | SU-MIMO | codebook-based Type 2 | random | 15 | 9 | 9 | 89.55% |  |
| Source 7 | R1-2110144 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 15 | 10.5 |  |  |  |
| Note 1: DL scheduler for dynamic grant based PDSCH scheduling: Delay aware (DA) |  |  |  |  |  |  |  |  |  |  |

Table B.1.2.2-4. FR1, DL, InH, CG 30Mbps, 60FPS, MU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 15 | 16.2 | 16 | 91.15% |  |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 15 | 16.67 | 16 | 92.01% | Note 1 |
| Source 3 | R1-2111234 | DDDSU | MU-MIMO | codebook-based Type 2 | random | 15 | 15 | 15 | 90% | Note 2, 3 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding |  | 15 | 12.9 | 12 | 90% | Note 2 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding |  | 15 | 13.3 | 13 | 92% | Note 2, 4 |
| Source 6 | R1-2111632 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 15 | 7.3 | 7 | 90.67% | Note 1 |
| Source 11 | R1-2111830 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 15 | 7.2 | 7 | 97.57% |  |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 15 | 12.8 | 12 | 95% |  |
| Source 7 | R1-2110144 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 15 | 12.3 |  |  |  |
| Note 1: DL scheduler for dynamic grant based PDSCH scheduling: Delay aware (DA) Note 2: 64QAM Note 3: Jitter STD=2ms, Jitter range Min=0ms, Jitter range Max=8ms Note 4: the traffic model for [3, 109, 91]% relationship |  |  |  |  |  |  |  |  |  |  |

    - Uma Scenario

      - VR/AR

        - Single stream traffic model

Table B.1.3.1.1-1. FR1, DL, Uma, VR/AR 30Mbps, 60FPS, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 9 | R1-2110811 | DDDSU | SU-MIMO | Close loop rank adaptation | random | 10 | 4.5 | 4 | 92.38% | Note 1 |
| Source 8 | R1-2110885 | DDDUU | SU-MIMO | Zeroforcing | random | 10 | 5.4 | 5 | 94% | Note 1 |
| Source 8 | R1-2110885 | DDDUU | SU-MIMO | cooperative MIMO/precoding | random | 10 | 6.5 | 6 | 93% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | SU-MIMO | Zeroforcing | random | 10 | 7 | 7 | 90% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | SU-MIMO | cooperative MIMO/precoding | random | 10 | 8.8 | 8 | 92% | Note 1 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 7.24 | 7 | 92.48% | Note 1 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 8.56 | 8 | 92.64% | Note 1, 3 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 11.7 | 11 | 95.40% | Note 1, 4 |
| Source 4 | R1-2108869 | DDDSU | SU-MIMO | reciprocity-based precoding | same | 10 | 2.98 | 2 | 98% | Note 2 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 4.4 | 4 | 94% | Note 1 |
| Source 5 | R1- 2112079 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 5.5 | 5 | 92.4% | Note 1 |
| Source 14 | R1-2112296 | DDDSU | SU-MIMO | codebook-based Type 2 | random | 10 | 8 | 8 | 89.05% | Note 1 |
| Source 7 | R1-2110144 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 7.2 |  |  | Note 1 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: BS antenna parameters: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1:8,2) Note 3: DL scheduler for dynamic grant based PDSCH scheduling: Delay aware (DA) Note 4: stream packet generation rate (Fps or Hz): 120 |  |  |  |  |  |  |  |  |  |  |

Table B.1.3.1.1-2. FR1, DL, Uma, VR/AR 30Mbps, 60FPS, MU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | Close loop rank adaptation | random | 10 | 9.3 | 9 | 91.22% | Note 1 |
| Source 8 | R1-2110885 | DDDUU | MU-MIMO | Zeroforcing | random | 10 | 6.3 | 6 | 91% | Note 1 |
| Source 8 | R1-2110885 | DDDUU | MU-MIMO | cooperative MIMO/precoding | random | 10 | 9.5 | 9 | 92% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | MU-MIMO | Zeroforcing | random | 10 | 7.7 | 7 | 94% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | MU-MIMO | cooperative MIMO/precoding | random | 10 | 11.6 | 11 | 92% | Note 1 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 8.82 | 8 | 93.75% | Note 1 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 9.55 | 9 | 92.30% | Note 1, 2 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 14.59 | 14 | 92.06% | Note 1, 3 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding |  | 10 | 10 | 10 | 90% | Note 1, 4 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 5.2 | 5 | 91% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO |  | random | 10 |  | 4 | 91% | Note 1, 5, 9 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO |  | random | 15 |  | 6 | 91% | Note 1, 6, 9 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO |  | random | 20 |  | 7 | 90% | Note 1, 7, 9 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO |  | random | 50 |  | 8 | 90% | Note 1, 8 ,9 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO |  | random | 10 |  | 5 | 91% | Note 1, 5, 10 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO |  | random | 15 |  | 7 | 90% | Note 1, 6, 10 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO |  | random | 20 |  | 7 | 92% | Note 1, 7, 10 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO |  | random | 50 |  | 8 | 91% | Note 1, 8 ,10 |
| Source 7 | R1-2110144 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 8.7 |  |  | Note 1 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: DL scheduler for dynamic grant based PDSCH scheduling: Delay aware (DA) Note 3: stream packet generation rate (Fps or Hz): 120 Note 4: 64QAM Note 5: ADU awareness, PDB=10ms: ADU capacity Note 6: ADU awareness, PDB=15ms: ADU capacity Note 7: ADU awareness, PDB=20ms: ADU capacity Note 8: ADU awareness, PDB=50ms: ADU capacity Note 9: 50ms packet discard time, capacity measured for AER target of 1% Note 10: 50ms packet discard time, capacity measured for PER target of 1% |  |  |  |  |  |  |  |  |  |  |

Table B.1.3.1.1-3. FR1, DL, Uma, VR/AR 45Mbps, 60FPS, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 9 | R1-2110811 | DDDSU | SU-MIMO | Close loop rank adaptation | random | 10 | 1.8 | 1 | 94.29% | Note 1 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 4.17 | 4 | 91.63% | Note 1 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 6.75 | 6 | 96.03% | Note 1, 3 |
| Source 4 | R1-2111360 | DDDSU | SU-MIMO | reciprocity-based precoding | same | 10 | 1.85 | 1 | 100% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 2.4 | 2 | 93% | Note 1 |
| Source 5 | R1- 2112079 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 4.7 | 4 | 92.7% | Note 1 |
| Source 14 | R1-2112296 | DDDSU | SU-MIMO | codebook-based Type 2 | random | 10 | 4.2 | 4 | 92.86% | Note 1 |
| Source 7 | R1-2110144 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 3.7 |  |  | Note 1 |
| Source 8 | R1-2108799 | DDDSU | SU-MIMO | Zeroforcing | random | 10 | 4.4 | 4 | 94% | Note 1 |
| Source 8 | R1-2108799 | DDDSU | SU-MIMO | cooperative MIMO/precoding | random | 10 | 5.4 | 5 | 93% | Note 1 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: BS antenna parameters: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1:8,2) Note 3: stream packet generation rate (Fps or Hz): 120 |  |  |  |  |  |  |  |  |  |  |

Table B.1.3.1.1-4. FR1, DL, Uma, VR/AR 45Mbps, 60FPS, MU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | Close loop rank adaptation | random | 10 | 4 | 4 | 90.00% | Note 1 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 4.68 | 4 | 94.05% | Note 1 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 8.12 | 8 | 90.87% | Note 1, 2 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding |  | 10 | 6 | 6 | 90% | Note 1, 3 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 2.9 | 2 | 93% | Note 1 |
| Source 7 | R1-2110144 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 10 | 4.6 |  |  | Note 1 |
| Source 8 | R1-2108799 | DDDSU | SU-MIMO | Zeroforcing | random | 10 | 4.9 | 4 | 96% | Note 1 |
| Source 8 | R1-2108799 | DDDSU | SU-MIMO | cooperative MIMO/precoding | random | 10 | 7.7 | 7 | 92% | Note 1 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: stream packet generation rate (Fps or Hz): 120 Note 3: 64QAM |  |  |  |  |  |  |  |  |  |  |

        - Multi-stream traffic model

Table B.1.3.1.2-1. FR1, DL, Uma, GOP-based 30Mbps, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 5 | R1-2112079 | DDDSU | SU-MIMO | reciprocity-based precoding | 1..5 | [10,10] | 4.2 | 4 | 90.65% | Note 1, 2 |
| Source 5 | R1-2112079 | DDDSU | SU-MIMO | reciprocity-based precoding | 2 | [10,10] | 2.4 | 2 | 92.85% | Note 1, 2 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: [PER_I, PER_P] = [1%, 1%] |  |  |  |  |  |  |  |  |  |  |

      - CG

Table B.1.3.2-1. FR1, DL, DU, CG 8Mbps, 60FPS, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 15 | 17.5 | 16 | 94% | Note 1 |
| Source 5 | R1- 2112079 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | >30 | >30 | 99% | Note 1 |
| Source 14 | R1-2112296 | DDDSU | SU-MIMO | codebook-based Type 2 | random | 15 | >20 | >20 | N/A | Note 1 |
| Source 7 | R1-2110144 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 15 | 32.9 |  |  | Note 1 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |  |  |  |  |  |  |  |  |  |  |

Table B.1.3.2-2. FR1, DL, DU, CG 8Mbps, 60FPS, MU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 15 | 23.8 | 23 | 93% | Note 1 |
| Source 7 | R1-2110144 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 15 | >36 |  |  | Note 1 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |  |  |  |  |  |  |  |  |  |  |

Table B.1.3.2-3. FR1, DL, Uma, CG 30Mbps, 60FPS, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 9 | R1-2110811 | DDDSU | SU-MIMO | Close loop rank adaptation | random | 15 | 6.5 | 6 | 92.86% | Note 1 |
| Source 8 | R1-2110885 | DDDUU | SU-MIMO | Zeroforcing | random | 15 | 7.2 | 7 | 91% | Note 1 |
| Source 8 | R1-2110885 | DDDUU | SU-MIMO | cooperative MIMO/precoding | random | 15 | 8.7 | 8 | 92% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | SU-MIMO | Zeroforcing | random | 15 | 9.7 | 9 | 92% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | SU-MIMO | cooperative MIMO/precoding | random | 15 | 11.4 | 11 | 91% | Note 1 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 15 | 10.33 | 10 | 91.90% | Note 1 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 15 | 11.94 | 11 | 93.78% | Note 1, 3 |
| Source 4 | R1-2108869 | DDDSU | SU-MIMO | reciprocity-based precoding | same | 15 | 4.08 | 4 | 90% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 15 | 5.4 | 5 | 92% | Note 1 |
| Source 5 | R1- 2112079 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 7.9 | 7 | 93.8% | Note 1 |
| Source 14 | R1-2112296 | DDDSU | SU-MIMO | codebook-based Type 2 | random | 15 | 9.5 | 9 | 92.35% |  |
| Source 7 | R1-2110144 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 15 | 9.2 |  |  | Note 1 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: BS antenna parameters: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1:8,2) Note 4: DL scheduler for dynamic grant based PDSCH scheduling: Delay aware (DA) |  |  |  |  |  |  |  |  |  |  |

Table B.1.3.2-4. FR1, DL, Uma, CG 30Mbps, 60FPS, MU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | Close loop rank adaptation | random | 15 | 12.4 | 12 | 92.46% | Note 1 |
| Source 8 | R1-2110885 | DDDUU | MU-MIMO | Zeroforcing | random | 15 | 8.4 | 8 | 91% | Note 1 |
| Source 8 | R1-2110885 | DDDUU | MU-MIMO | cooperative MIMO/precoding | random | 15 | 12.4 | 12 | 91% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | MU-MIMO | Zeroforcing | random | 15 | 11.1 | 11 | 90% | Note 1 |
| Source 8 | R1-2110885 | DDDSU | MU-MIMO | cooperative MIMO/precoding | random | 15 | 14.2 | 14 | 91% | Note 1 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 15 | 14.33 | 14 | 91.33% | Note 1 |
| Source 18 | R1-2111046 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 15 | 14.45 | 14 | 91.73% | Note 1, 2 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding |  | 15 | 11.6 | 11 | 93% | Note 1, 3 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 15 | 8 | 8 | 90% | Note 1 |
| Source 7 | R1-2110144 | DDDSU | MU-MIMO | reciprocity-based precoding | random | 15 | 12.1 |  |  | Note 1 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: DL scheduler for dynamic grant based PDSCH scheduling: Delay aware (DA) Note 3: 64QAM |  |  |  |  |  |  |  |  |  |  |

  - FR1 UL

    - DU Scenario

      - VR/CG (Pose/control-stream)

Table B.2.1.1-1. FR1, UL, DU, VR/CG 0.2Mbps, 250FPS, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 8 | R1-2110885 | DDDUU | SU-MIMO | single layer transmission | evenly spaced | 10 | 178.4 | 178 | 90% | Note 1 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 20 | 20 | 99.99% | Note 1 |
| Source 15 | R1-2112572 | DDDSU | SU-MIMO |  | random | 10 | 45.77 | 45 | 98% | Note 2 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | codebook-based Type 1 | random | 10 | 224.9 | 224 | 92% | Note 1 |
| Source 14 | R1-2109555 | DDDSU | SU-MIMO | codebook-based Type 2 | random | 10 | >30 | >30 | 100% | Note 1 |
| Source 7 | R1-2110144 | DDDUU | SU-MIMO | reciprocity-based precoding | random | 10 | 39.9 |  |  | Note 1 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: BS antenna parameters: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1:8,2) |  |  |  |  |  |  |  |  |  |  |

Table B.2.1.1-2. FR1, UL, DU, VR/CG 0.2Mbps, 250FPS, MU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | Close loop rank adaptation | random | 10 | >15 |  | 100% (15) | Note 1 |
| Source 11 | R1-2111830 | DDDSU | MU-MIMO | 32-port CSI-RS Type I codebook | evenly spaced | 10 | 8 | 8 | 96.50% | Note 2 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | codebook-based Type 1 | random | 10 | >240 | 240 | 99% | Note 1 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: BS antenna parameters: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1:8,2) |  |  |  |  |  |  |  |  |  |  |

      - AR (1 stream: Scene/video/data/voice-stream)

Table B.2.1.2-1. FR1, UL, DU, AR (1 stream: Scene/video/data/voice-stream), 10Mbps, 60FPS, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 30 | 9.49 | 9 | 92.95% | Note 1 |
| Source 10 | R1-2111521 | DDDSU | SU-MIMO |  | random | 30 | 7.8 | 7 | 98.23 | Note 1, 3 |
| Source 10 | R1-2111521 | DDDSU | SU-MIMO |  | random | 30 | 7.81 | 7 | 98.09 | Note 1 |
| Source 15 | R1-2112572 | DDDSU | SU-MIMO |  | random | 30 | 4.77 | 4 | 91% | Note 2 |
| Source 7 | R1-2112551 | DDDUU | SU-MIMO | reciprocity-based precoding | random | 30 | 7.5 | 7 |  | Note 1 |
| Source 7 | R1-2112551 | DDDUU | SU-MIMO | reciprocity-based precoding | random | 30 | 8.4 | 8 |  | Note 1, 4 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | codebook-based Type 1 | random | 30 | 4.5 | 4 | 93.3% | Note 1 |
| Source 14 | R1-2112296 | DDDSU | SU-MIMO | codebook-based Type 2 | random | 30 | 9.39 | 9 | 90% | Note 1 |
| Source 8 | R1-2110885 | DDDUU | SU-MIMO | single layer transmission | random | 30 | 7.4 | 7 | 93% | Note 1 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: BS antenna parameters: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1:8,2) Note 3: Target BLER 1% Note 4: Elastic BSR |  |  |  |  |  |  |  |  |  |  |

Table B.2.1.2-2. FR1, UL, DU, AR (1 stream: Scene/video/data/voice-stream), 10Mbps, 60FPS, MU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | Close loop rank adaptation | random | 30 | 8.1 | 8 | 91.67% | Note 1 |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | Close loop rank adaptation | random | 10 | <1 |  |  | Note 1 |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | Close loop rank adaptation | random | 15 | 5.4 | 5 | 92.19% | Note 1 |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | Close loop rank adaptation | random | 60 | 8.3 | 8 | 93.81% | Note 1 |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | Close loop rank adaptation | random | 30 | 8.3 | 8 | 93.10% | Note 1, 4 |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | Close loop rank adaptation | random | 30 | 8.4 | 8 | 94.05% | Note 1, 5 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding |  | 30 | 10.9 | 10 | 94% | Note1, 6 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding |  | 30 | 9.5 | 9 | 95% | Note1, 6, 7 |
| Source 10 | R1-2111521 | DDDSU | MU-MIMO |  | random | 30 | 10.49 | 10 | 95.24 | Note 1, 8 |
| Source 10 | R1-2111521 | DDDSU | MU-MIMO |  | random | 30 | 10.5 | 10 | 95.29 | Note 1 |
| Source 11 | R1-2111830 | DDDSU | MU-MIMO | 32-port CSI-RS Type I codebook | random | 30 | 2.3 | 2 | 96% | Note 2, 3 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | codebook-based Type 1 | random | 30 | 7.3 | 7 | 90% | Note 1 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: BS antenna parameters: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1:8,2) Note 3: with jitter Note 4: X=95 Note 5: X =90 Note 6: 64QAM Note 7: legacy BSR Note 8: Target BLER 1% |  |  |  |  |  |  |  |  |  |  |

Table B.2.1.2-3. FR1, UL, DU, AR (1 stream: Scene/video/data/voice-stream), 20Mbps, 60FPS, MU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding |  | 30 | 3.4 | 3 | 91% | Note 1, 2, 3 |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding |  | 30 | 5.1 | 5 | 90% | Note 1, 2, 4 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: 64QAM Note 3: legacy BSR Note 4: Enhanced BSR |  |  |  |  |  |  |  |  |  |  |

      - AR (2 streams: Pose/control-stream + scene/video/data/voice-stream)

Table B.2.1.3-1. FR1, UL, DU, AR (2 streams: Pose/control-stream + scene/video/data/voice-stream), 10.2Mbps, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10; 30 | 7.43 | 7 | 92.29% | Note 1 |
| Source 10 | R1-2111521 | DDDSU | SU-MIMO |  | random | 10; 30 | 3.35 | 3 | 91.9 | Note 1, 2 |
| Source 10 | R1-2111521 | DDDSU | SU-MIMO |  | random | 10; 30 | 3.41 | 3 | 91.58 | Note 1 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | codebook-based Type 1 | random | 10; 30 | 4.1 | 4 | 90.4% | Note 1 |
| Source 7 | R1-2110144 | DDDUU | SU-MIMO | reciprocity-based precoding | random | 10 | 2.6 |  |  | Note 1 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: Target BLER 1% |  |  |  |  |  |  |  |  |  |  |

Table B.2.1.3-2. FR1, UL, DU, AR (2 streams: Pose/control-stream + scene/video/data/voice-stream), 10.2Mbps, MU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | Close loop rank adaptation | random | 10; 30 | 1.5 | 1 | 92.38% | Note 1 |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | Close loop rank adaptation | random | 10; 30 | 5.6 | 5 | 94.48% | Note 1, 3 |
| Source 10 | R1-2111521 | DDDSU | MU-MIMO |  | random | 10; 30 | 4.57 | 4 | 90.75 | Note 1, 4 |
| Source 10 | R1-2111521 | DDDSU | MU-MIMO |  | random | 10; 30 | 4.91 | 4 | 90.98 | Note 1 |
| Source 11 | R1-2111830 | DDDSU | MU-MIMO | 32-port CSI-RS Type I codebook | random | 10; 30 | 0 | 0 | 0% | Note 2, 5 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | codebook-based Type 1 | random | 10; 30 | 5.8 | 5 | 92.4% | Note 1 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: BS antenna parameters: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1:8,2) Note 3: Aware-traffic Note 4: Target BLER 1% Note 5: video-stream with jitter |  |  |  |  |  |  |  |  |  |  |

      - AR (3 streams: Video stream+Data/audio stream+Pose/control stream)

Table B.2.1.4-1. FR1, UL, DU, AR (3 streams: Video stream 10Mbps+Data/audio stream 1.12Mbps+Pose/control stream 0.2Mbps), SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 1 | R1-2111902 | DDDSU | SU-MIMO |  |  | 10; 30; 10 | 3 | 3 |  | Note 1 |
| Note 1: BS antenna parameters: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1,8,2) |  |  |  |  |  |  |  |  |  |  |

      - AR (3 streams: Pose/control-stream + I/P-stream)

Table B.2.1.5-1. FR1, UL, DU, AR (3 streams: Pose/control-stream + I/P-stream with α = 2) 10.2Mbps, MU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | Close loop rank adaptation | random | 10; 30; 30 | 3.5 | 3 | 92.06% | Note 1 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |  |  |  |  |  |  |  |  |  |  |

    - InH Scenario

      - VR/CG (Pose/control-stream)

Table B.2.2.1-1. FR1, UL, InH, VR/CG 0.2Mbps, 250FPS, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 20 | 20 | 100.00% |  |
| Source 3 | R1-2111234 | DDDUU | SU-MIMO | codebook-based Type 2 | random | 10 | >12 | >12 |  | Note 1 |
| Source 15 | R1-2112572 | DDDSU | SU-MIMO |  | random | 10 | 54.59 | 54 | 97% |  |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | codebook-based Type 1 | random | 10 | 198 | 192 | 99% |  |
| Source 14 | R1-2112296 | DDDSU | SU-MIMO | codebook-based Type 2 | random | 10 | >30 | >30 | 100% |  |
| Source 7 | R1-2110144 | DDDUU | SU-MIMO | reciprocity-based precoding | random | 10 | >40 |  |  |  |
| Note 1: 64QAM |  |  |  |  |  |  |  |  |  |  |

Table B.2.2.1-2. FR1, UL, InH, VR/CG 0.2Mbps, 250FPS, MU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 20 | R1-2111351 | DDDSU | MU-MIMO | reciprocity-based precoding |  | 10 | >40 | 40 | 100% | Note 1 |
| Source 11 | R1-2111830 | DDDSU | MU-MIMO | 32-port CSI-RS Type I codebook | evenly spaced | 10 | 20 | 20 | 100% | Note 2 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | codebook-based Type 1 | random | 10 | >240 | 240 | 99% |  |
| Note 1: 64QAM Note 2: with jitter |  |  |  |  |  |  |  |  |  |  |

      - AR (1 stream: Scene/video/data/voice-stream)

Table B.2.2.2-1. FR1, UL, InH, AR (1 stream: Scene/video/data/voice-stream), 10Mbps, 60FPS, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 30 | 13.95 | 13 | 93.59% |  |
| Source 3 | R1-2111234 | DDDUU | SU-MIMO | codebook-based Type 2 | random | 30 | 6 | 6 | 100% | Note 1 |
| Source 15 | R1-2112572 | DDDSU | SU-MIMO |  | random | 30 | 4.66 | 4 | 99% |  |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | codebook-based Type 1 | random | 30 | 4.4 | 4 | 97.3% |  |
| Source 14 | R1-2112296 | DDDSU | SU-MIMO | codebook-based Type 2 | random | 30 | 5.09 | 5 | 90% |  |
| Source 7 | R1-2110144 | DDDUU | SU-MIMO | reciprocity-based precoding | random | 30 | 6.1 |  |  |  |
| Note 1: 64QAM |  |  |  |  |  |  |  |  |  |  |

Table B.2.2.2-2. FR1, UL, InH, AR (1 stream: Scene/video/data/voice-stream), 10Mbps, 60FPS, MU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 11 | R1-2111830 | DDDSU | MU-MIMO | 32-port CSI-RS Type I codebook | random | 30 | 11.5 | 11 | 94.50% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | codebook-based Type 1 | random | 30 | 7.1 | 7 | 95% |  |
| Note 1: video-stream with jitter |  |  |  |  |  |  |  |  |  |  |

      - AR (2 streams: Pose/control-stream + scene/video/data/voice-stream)

Table B.2.2.3-1. FR1, UL, InH, AR (2 streams: Pose/control-stream + scene/video/data/voice-stream), 10.2Mbps, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10; 30 | 12.71 | 12 | 93.29% |  |
| Source 15 | R1-2112572 | DDDSU | SU-MIMO |  | random | 10; 10 | 4.05 | 4 | 94% |  |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | codebook-based Type 1 | random | 10; 30 | 4.1 | 4 | 91.9% |  |
| Source 7 | R1-2110144 | DDDUU | SU-MIMO | reciprocity-based precoding | random | 10; 30 | 5.8 |  |  |  |

Table B.2.2.3-2. FR1, UL, InH, AR (2 streams: Pose/control-stream + scene/video/data/voice-stream), 10.2Mbps, MU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 11 | R1-2111830 | DDDSU | MU-MIMO | 32-port CSI-RS Type I codebook | random | 10; 30 | 7.2 | 7 | 94% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | codebook-based Type 1 | random | 10; 30 | 7.4 | 7 | 95.4% |  |
| Note 1: video-stream with jitter |  |  |  |  |  |  |  |  |  |  |

      - AR (3 streams: Video stream+Data/audio stream+Pose/control stream)

Table B.2.2.4-1. FR1, UL, InH, AR (3 streams: Video stream 10Mbps+Data/audio stream 1.12Mbps+Pose/control stream 0.2Mbps), SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 1 | R1-2111902 | DDDSU | SU-MIMO |  |  | 10; 30; 10 | 4.1 | 4 | 91% |  |

    - Uma Scenario

      - VR/CG (Pose/control-stream)

Table B.2.3.1-1. FR1, UL, Uma, VR/CG 0.2Mbps, 250FPS, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 8 | R1-2110885 | DDDUU | SU-MIMO | single layer transmission | evenly spaced | 10 | 142.4 | 142 | 95% | Note 1 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 20 | 20 | 97.70% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | codebook-based Type 1 | random | 10 | 143 | 136 | 94% | Note 1 |
| Source 14 | R1-2112296 | DDDSU | SU-MIMO | codebook-based Type 2 | random | 10 | >30 | >30 | 100% | Note 1, 2 |
| Source 7 | R1-2110144 | DDDUU | SU-MIMO | reciprocity-based precoding | random | 10 | 17.4 |  |  | Note 1 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: downtilt: 12 |  |  |  |  |  |  |  |  |  |  |

Table B.2.3.1-2. FR1, UL, Uma, VR/CG 0.2Mbps, 250FPS, MU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | Close loop rank adaptation | random | 10 | >15 |  | 95.56% (15) | Note 1, 2 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | codebook-based Type 1 | random | 10 | >240 | 240 | 93% | Note 1 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: downtilt: 12 |  |  |  |  |  |  |  |  |  |  |

      - AR (1 stream: Scene/video/data/voice-stream)

Table B.2.3.2-1. FR1, UL, Uma, AR (1 stream: Scene/video/data/voice-stream), 10Mbps, 60FPS, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 8 | R1-2110885 | DDDUU | SU-MIMO | single layer transmission | random | 30 | <1 | 0 | 100% | Note 1 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 30 | <1 | 0 | 74.60% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | codebook-based Type 1 | random | 30 | 0 | 0 | N.A. | Note 1 |
| Source 14 | R1-2112296 | DDDSU | SU-MIMO | codebook-based Type 2 | random | 30 | 1.34 | 1 | 90% | Note 1, 2 |
| Source 7 | R1-2110144 | DDDUU | SU-MIMO | reciprocity-based precoding | random | 30 | <1 |  |  | Note 1 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: downtilt: 12 |  |  |  |  |  |  |  |  |  |  |

Table B.2.3.2-2. FR1, UL, Uma, AR (1 stream: Scene/video/data/voice-stream), 10Mbps, 60FPS, MU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 9 | R1-2110811 | DDDSU | MU-MIMO | Close loop rank adaptation | random | 30 | <1 |  |  | Note 1, 2 |
| Source 16 | R1-2112720 | DDDSU | MU-MIMO | codebook-based Type 1 | random | 30 | 0 | 0 | 0% | Note 1 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) Note 2: downtilt: 12 |  |  |  |  |  |  |  |  |  |  |

      - AR (2 streams: Pose/control-stream + scene/video/data/voice-stream)

Table B.2.3.3-1. FR1, UL, Uma, AR (2 streams: Pose/control-stream + scene/video/data/voice-stream), 10.2Mbps, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | codebook-based Type 1 | random | 10; 30 | 0 | 0 | N.A. | Note 1 |
| Source 7 | R1-2110144 | DDDUU | SU-MIMO | reciprocity-based precoding | random | 10; 30 | <1 |  |  | Note 1 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |  |  |  |  |  |  |  |  |  |  |

Table B.2.3.3-2. FR1, UL, Uma, AR (2 streams: Pose/control-stream + scene/video/data/voice-stream), 10.2Mbps, MU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO | codebook-based Type 1 | random | 10; 30 | 0 | 0 | N.A. | Note 1 |
| Note 1: BS antenna parameters: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |  |  |  |  |  |  |  |  |  |  |

  - FR2 DL

    - DU Scenario

      - VR/AR

        - Single stream traffic model

Table B.3.1.1.1-1. FR2, DL, DU, VR/AR 30Mbps, 60FPS, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 13.44 | 13 | 95.24% |  |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 14.16 | 14 | 91.27% | Note 1, 2 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 16.28 | 16 | 93.55% | Note 1, 3 |
| Source 15 | R1-2112572 | DDDSU | SU-MIMO |  | random | 10 | 6.35 | 6 | 96% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 8.5 | 8 | 91% | Note 1, 5 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 4 | 4 | 90% | Note 1, 6 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 8.5 | 8 | 91% | Note 1, 7 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 0 | 0 | n/a | Note 1, 8 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 3 | 3 | 90% | Note 1, 9 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 5 | 5 | 90% | Note 1, 10 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 14.5 | 14 | 92% | Note 1, 11 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 15 | 15 | 90% | Note 1, 12 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 0 | 0 | n/a | Note 13, |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 10 | 10 | 90% | Note 14, |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 7 | 7 | 90% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | Synch | 10 | 6 | 6 | 90% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 7 | 7 | 90% | Note 1,3 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 30 | 30 | 90% | Note 1, 4 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | Synch | 10 | 22.5 | 22 | 91% | Note 1, 4 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 20.5 | 20 | 92% | Note 1, 3, 4 |
| Source 16 | R1-2112720 | DDDUU | SU-MIMO |  | random | 10 | 5.5 | 5 | 93% | Note 1 |
| Source 16 | R1-2112720 | DDDDU | SU-MIMO |  | random | 10 | 21.5 | 21 | 92% | Note 1, 4 |
| Source 14 | R1-2112296 | DDDSU | SU-MIMO | codebook-based Type 2 | random | 10 | 10 | 10 | 88.58% | Note 15 |
| Source 7 | R1-2110144 | DDDUU | SU-MIMO | codebook-based Type 1 | random | 10 | 4.2 |  |  | Note 1 |
| Note 1: UE antenna configuraiton: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) Note 2: Delay aware (DA) scheduler Note 3: stream packet generation rate (Fps or Hz): 120 Note 4: 400MHz bandwidth Note 5: baseline, 2CC(30&39GHz) CA, no blocking Note 6: PDCP duplication, 2CC(30&39GHz) CA, no blocking Note 7: network coding(50% redundancy), 2CC(30&39GHz) CA, no blocking Note 8: baseline, 2CC(30&39GHz) CA, periodic blocking(4/10ms) on 30GHz CC Note 9: PDCP duplication, 2CC(30&39GHz) CA, periodic blocking(4/10ms) on 30GHz CC Note 10: network coding(100% redundancy), 2CC(30&39GHz) CA, periodic blocking(4/10ms) on 30GHz CC Note 11: baseline, 4CC(30,30.4,39&39.4GHz) CA, no blocking Note 12: network coding(20% redundancy), 4CC(30,30.4,39&39.4GHz) CA, no blocking Note 13: baseline, 4CC(30,30.4,39&39.4GHz) CA, periodic blocking (4/10ms) on 39&39.4GHz CCs Note 14: network coding(120% redundancy), 4CC(30,30.4,39&39.4GHz) CA, periodic blocking (4/10ms) on 39&39.4GHz CCs Note 15: UE antenna configuraiton: 4Tx/4Rx: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2) |  |  |  |  |  |  |  |  |  |  |

Table B.3.1.1.1-2. FR2, DL, DU, VR/AR 45Mbps, 60FPS, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 8.2 | 8 | 93.25% | Note 1 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 10.32 | 10 | 93.97% | Note 1, 2 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 43.89 | 43 | 91.92% | Note 1, 3 |
| Source 15 | R1-2112572 | DDDSU | SU-MIMO |  | random | 10 | 3.94 | 3 | 98% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 4.5 | 4 | 91% | Note 1, 4 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 2.5 | 2 | 94% | Note 1, 5 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 5 | 5 | 90% | Note 1, 6 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 0 | 0 | n/a | Note 1, 7 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 2 | 2 | 89% | Note 1, 8 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 3 | 3 | 89% | Note 1, 9 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 10 | 10 | 92% | Note 1, 10 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 10 | 10 | 92% | Note 1, 11 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 0 | 0 | n/a | Note 1, 12 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 6 | 6 | 90% | Note 1, 13 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 10.5 | 10 | 92% | Note 1, 14 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 9 | 9 | 90% | Note 1, 15 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 5 | 5 | 91% | Note 1, 16 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 5 | 5 | 90% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | Synch | 10 | 3.5 | 3 | 92% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 5.5 | 5 | 93% | Note 1,2 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 22.5 | 22 | 93% | Note 1, 3 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | Synch | 10 | 18 | 18 | 90% | Note 1, 3 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 17.5 | 17 | 92% | Note 1, 2,3 |
| Source 16 | R1-2112720 | DDDUU | SU-MIMO |  | random | 10 | 2.5 | 2 | 93% | Note 1 |
| Source 16 | R1-2112720 | DDDDU | SU-MIMO |  | random | 10 | 16.5 | 16 | 90% | Note 1, 3 |
| Source 14 | R1-2112296 | DDDSU | SU-MIMO | codebook-based Type 2 | random | 10 | 4.7 | 4 | 92.62% | Note 17 |
| Source 7 | R1-2110144 | DDDUU | SU-MIMO | codebook-based Type 1 | random | 10 | 2 |  |  | Note 1 |
| Note 1: UE antenna configuraiton: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) Note 2: stream packet generation rate (Fps or Hz): 120 Note 3: 400MHz bandwidth Note 4: baseline, 2CC(30&39GHz) CA, no blocking Note 5: PDCP duplication, 2CC(30&39GHz) CA, no blocking Note 6: network coding(20% redundancy), 2CC(30&39GHz) CA, no blocking Note 7: baseline, 2CC(30&39GHz) CA, periodic blocking(4/10ms) on 30GHz CC Note 8: PDCP duplication, 2CC(30&39GHz) CA, periodic blocking(4/10ms) on 30GHz CC Note 9: network coding(100% redundancy), 2CC(30&39GHz) CA, periodic blocking(4/10ms) on 30GHz CC Note 10: baseline, 4CC(30,30.4,39&39.4GHz) CA, no blocking Note 11: network coding(20% redundancy), 4CC(30,30.4,39&39.4GHz) CA, no blocking Note 12: baseline, 4CC(30,30.4,39&39.4GHz) CA, periodic blocking (4/10ms) on 39&39.4GHz CCs Note 13: network coding(120% redundancy), 4CC(30,30.4,39&39.4GHz) CA, periodic blocking (4/10ms) on 39&39.4GHz CCs Note 14: network coding (100% redundancy), mTRP (2ms evaluation interval), periodic blocking (4/10ms) with probability 0.2 Note 15: network coding (100% redundancy), mTRP (2ms evaluation interval),periodic blocking (40/100ms) with probability 0.2 Note 16: network coding (100% redundancy), mTRP (10ms evaluation interval), periodic blocking (40/100ms) with probability 0.2Note 17: UE antenna configuraiton: 4Tx/4Rx: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2) |  |  |  |  |  |  |  |  |  |  |

        - Multi-stream traffic model

Table B.3.1.1.2-1. FR2, DL, DU, 2 stream: VR 30Mbps+audio-stream 0.756Mbps, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | [Video_PDB, Audio_PDB] (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | [10, 30] | 6 | 6 | 90% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | [10, 30] | 6.5 | 6 | 93% | Note 1, 2 |
| Source 16 | R1-2112720 | DDDDU | SU-MIMO |  | random | [10, 30] | 3.5 | 3 | 92% | Note 1 |
| Note 1: UE antenna configuraiton: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) Note 2: Delay aware (DA) scheduler |  |  |  |  |  |  |  |  |  |  |

      - CG

Table B.3.1.2-1. FR2, DL, DU, CG 8Mbps, 60FPS, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 15 | 32.5 | 32 | 94% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 15 | >45 | >45 | N/A | Note 1, 3 |
| Source 14 | R1-2112296 | DDDSU | SU-MIMO | codebook-based Type 2 | random | 15 | >20 | >20 | N/A | Note 2 |
| Note 1: UE antenna configuraiton: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) Note 2: UE antenna configuraiton: 4Tx/4Rx: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2) Note 3: 400MHz bandwidth |  |  |  |  |  |  |  |  |  |  |

Table B.3.1.2-2. FR2, DL, DU, CG 30Mbps, 60FPS, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 15 | 16.16 | 16 | 92.36% | Note 1 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 15 | 16.82 | 16 | 96.73% | Note 1, 3 |
| Source 15 | R1-2112572 | DDDSU | SU-MIMO |  | random | 15 | 8.25 | 8 | 93% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 15 | 8 | 8 | 90% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 15 | 32.5 | 32 | 93% | Note 1, 4 |
| Source 14 | R1-2112296 | DDDSU | SU-MIMO | codebook-based Type 2 | random | 15 | 11 | 11 | 90.60% | Note 2 |
| Source 7 | R1-2110144 | DDDUU | SU-MIMO | codebook-based Type 1 | random | 15 | 5.1 |  |  | Note 1 |
| Note 1: UE antenna configuraiton: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) Note 2: UE antenna configuraiton: 4Tx/4Rx: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2) Note 3: Delay aware (DA) scheduler Note 4: 400MHz bandwidth |  |  |  |  |  |  |  |  |  |  |

    - InH Scenario

      - VR/AR

        - Single stream traffic model

Table B.3.2.1.1-1. FR2, DL, InH, VR/AR 30Mbps, 60FPS, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 8.72 | 8 | 92.01% | Note 1 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 8.83 | 8 | 92.36% | Note1, 3 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 10.23 | 10 | 91.94% | Note 1, 4 |
| Source 20 | R1-2111351 | DDDSU | SU-MIMO | reciprocity-based precoding |  | 10 | 7.8 | 7 | 91% | Note 2, 5 |
| Source 15 | R1-2112572 | DDDSU | SU-MIMO |  | random | 10 | 10.17 | 10 | 98% | Note 1 |
| Source 7 | R1-2112551 | DDDSU | SU-MIMO | codebook-based Type 1 | random | 10 | 6.2 | 6 |  | Note 1 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 7 | 7 | 90% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | Synch | 10 | 6.5 | 6 | 91% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 7.5 | 7 | 92% | Note 1,4 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 34 | 34 | 90% | Note 1, 6 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | Synch | 10 | 26.5 | 26 | 92% | Note 1, 6 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 21.5 | 21.0 | 91% | Note 1, 4, 6 |
| Source 16 | R1-2112720 | DDDUU | SU-MIMO |  | random | 10 | 5.5 | 5 | 93% | Note 1 |
| Source 16 | R1-2112720 | DDDUU | SU-MIMO |  | random | 10 | 25 | 25 | 90% | Note 1, 6 |
| Source 14 | R1-2112296 | DDDSU | SU-MIMO | codebook-based Type 2 | random | 10 | 10 | 10 | 89.00% | Note 2 |
| Note 1: UE antenna configuraiton: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) Note 2: UE antenna configuraiton: 4Tx/4Rx: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2) Note 3: Delay aware (DA) scheduler Note 4: stream packet generation rate (Fps or Hz): 120 Note 5: 64QAM Note 6: 400MHz bandwidth |  |  |  |  |  |  |  |  |  |  |

Table B.3.2.1.1-2. FR2, DL, InH, VR/AR 45Mbps, 60FPS, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 4.67 | 4 | 94.44% | Note 1 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 6.03 | 6 | 90.28% | Note 1, 3 |
| Source 15 | R1-2112572 | DDDSU | SU-MIMO |  | random | 10 | 6.09 | 6 | 98% | Note 1 |
| Source 7 | R1-2112551 | DDDSU | SU-MIMO | codebook-based Type 1 | random | 10 | 3.2 | 3 |  | Note 1 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 5 | 5 | 90% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | Synch | 10 | 4 | 4 | 90% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 5.5 | 5 | 92% | Note 1,3 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 27 | 27 | 90% | Note 1, 4 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | Synch | 10 | 21 | 21 | 90% | Note 1, 4 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 18.5 | 18 | 92% | Note 1, 3,4 |
| Source 16 | R1-2112720 | DDDUU | SU-MIMO |  | random | 10 | 2.5 | 2 | 93% | Note 1 |
| Source 16 | R1-2112720 | DDDUU | SU-MIMO |  | random | 10 | 19 | 19 | 90% | Note 1, 4 |
| Source 14 | R1-2112296 | DDDSU | SU-MIMO | codebook-based Type 2 | random | 10 | 4.7 | 4 | 96.26% | Note 2 |
| Note 1: UE antenna configuraiton: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) Note 2: UE antenna configuraiton: 4Tx/4Rx: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2) Note 3: stream packet generation rate (Fps or Hz): 120 Note 4: 400MHz bandwidth |  |  |  |  |  |  |  |  |  |  |

        - Multi-stream traffic model

Table B.3.2.1.2-1. FR2, DL, InH, 2 stream: I/P Frame Traffic Model GOP-Based, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | [I_PDB, P_PDB] (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 1.5 | [10,10] | 5.37 | 5 | 91.20% | Note 1,2 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 1.5 | [10,10] | 5.43 | 5 | 91.55% | Note 1,3 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 1.5 | [10,10] | 4.98 | 4 | 93.75% | Note 1,4 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 1.5 | [15,10] | 7.07 | 7 | 90.34% | Note 1,2 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 1.5 | [15,10] | 7.43 | 7 | 91.61% | Note 1,3 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 1.5 | [15,10] | 6.8 | 6 | 93.06% | Note 1,4 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 1.5 | [15,9] | 6.91 | 6 | 93.98% | Note 1,2 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 1.5 | [15,9] | 7.11 | 7 | 90.56% | Note 1,3 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 1.5 | [15,9] | 6.93 | 6 | 94.44% | Note 1,4 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 2 | [10,10] | 3.53 | 3 | 92.01% | Note 1,2 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 2 | [10,10] | 3.87 | 3 | 92.71% | Note 1,3 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 2 | [10,10] | 2.73 | 2 | 93.06% | Note 1,4 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 2 | [15,10] | 5.23 | 5 | 91.15% | Note 1,2 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 2 | [15,10] | 5.52 | 5 | 92.71% | Note 1,3 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 2 | [15,10] | 4.91 | 4 | 94.94% | Note 1,4 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 2 | [15,9] | 4.99 | 4 | 94.68% | Note 1,2 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 2 | [15,9] | 5.33 | 5 | 91.67% | Note 1,3 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 2 | [15,9] | 4.78 | 4 | 94.14% | Note 1,4 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 3 | [10,10] | 2.29 | 2 | 93.06% | Note 1,2 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 3 | [10,10] | 2.29 | 2 | 93.06% | Note 1,3 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 3 | [10,10] | 2.03 | 2 | 90.28% | Note 1,4 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 3 | [15,10] | 3.29 | 3 | 91.32% | Note 1,2 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 3 | [15,10] | 3.29 | 3 | 91.32% | Note 1,3 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 3 | [15,10] | 2.68 | 2 | 93.06% | Note 1,4 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 3 | [15,9] | 3.29 | 3 | 90.97% | Note 1,2 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 3 | [15,9] | 3.29 | 3 | 90.97% | Note 1,3 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 3 | [15,9] | 2.68 | 2 | 93.06% | Note 1,4 |
| Note 1: UE antenna configuraiton: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) Note 2: [PER_I, PER_P] = [1%, 1%] Note 3: [PER_I, PER_P] = [1%, 5%] Note 4: [PER_I, PER_P] = [0.5%, 5%] |  |  |  |  |  |  |  |  |  |  |

Table B.3.2.1.2-2. FR2, DL, InH, 2 stream: I/P Frame Traffic Model Slice-Based, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | [I_PDB, P_PDB] (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 1.5 | [10,10] | 8.23 | 8 | 92.53% | Note 1,2 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 1.5 | [10,10] | 10.61 | 10 | 92.08% | Note 1,3 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 1.5 | [10,10] | 10.46 | 10 | 91.67% | Note 1,4 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 1.5 | [15,10] | 8.24 | 8 | 92.71% | Note 1,2 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 1.5 | [15,10] | 10.77 | 10 | 92.50% | Note 1,3 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 1.5 | [15,10] | 10.55 | 10 | 91.94% | Note 1,4 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 1.5 | [15,9] | 8.14 | 8 | 91.67% | Note 1,2 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 1.5 | [15,9] | 10.51 | 10 | 91.48% | Note 1,3 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 1.5 | [15,9] | 10.43 | 10 | 91.39% | Note 1,4 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 2 | [10,10] | 8.24 | 8 | 92.71% | Note 1,2 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 2 | [10,10] | 10.73 | 10 | 92.50% | Note 1,3 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 2 | [10,10] | 10.46 | 10 | 91.67% | Note 1,4 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 2 | [15,10] | 8.24 | 8 | 92.71% | Note 1,2 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 2 | [15,10] | 10.72 | 10 | 92.50% | Note 1,3 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 2 | [15,10] | 10.66 | 10 | 92.22% | Note 1,4 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 2 | [15,9] | 8.18 | 8 | 92.01% | Note 1,2 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 2 | [15,9] | 10.38 | 10 | 91.39% | Note 1,3 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 2 | [15,9] | 10.45 | 10 | 91.53% | Note 1,4 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 3 | [10,10] | 8.23 | 8 | 92.53% | Note 1,2 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 3 | [10,10] | 10.61 | 10 | 92.08% | Note 1,3 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 3 | [10,10] | 10.38 | 10 | 91.39% | Note 1,4 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 3 | [15,10] | 8.28 | 8 | 93.06% | Note 1,2 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 3 | [15,10] | 10.63 | 10 | 92.22% | Note 1,3 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 3 | [15,10] | 10.55 | 10 | 91.94% | Note 1,4 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 3 | [15,9] | 8.22 | 8 | 92.36% | Note 1,2 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 3 | [15,9] | 10.46 | 10 | 91.49% | Note 1,3 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | random | 3 | [15,9] | 10.48 | 10 | 91.67% | Note 1,4 |
| Note 1: UE antenna configuraiton: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) Note 2: [PER_I, PER_P] = [1%, 1%] Note 3: [PER_I, PER_P] = [1%, 5%] Note 4: [PER_I, PER_P] = [0.5%, 5%] |  |  |  |  |  |  |  |  |  |  |

Table B.3.2.1.2-3. FR2, DL, InH, 2 stream: VR 30Mbps+audio-stream 0.756Mbps, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | [Video_PDB, Audio_PDB] (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | [10,30] | 6 | 6 | 90% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | [10,30] | 7 | 7 | 90% | Note 1, 2 |
| Source 16 | R1-2112720 | DDDDU | SU-MIMO |  | random | [10,30] | 4 | 4 | 90% | Note 1 |
| Note 1: UE antenna configuraiton: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) Note 2: Delay aware (DA) scheduler |  |  |  |  |  |  |  |  |  |  |

      - CG

Table B.3.2.2-1. FR2, DL, InH, CG 8Mbps, 60FPS, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 7 | R1-2112551 | DDDSU | SU-MIMO | codebook-based Type 1 | random | 15 | 28 | 28 |  | Note 1 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 15 | 31 | 31 | 90% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 15 | 44 | 44 | 90% | Note 1, 3 |
| Source 14 | R1-2112296 | DDDSU | SU-MIMO | codebook-based Type 2 | random | 15 | >20 | >20 | N/A | Note 2 |
| Note 1: UE antenna configuraiton: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) Note 2: UE antenna configuraiton: 4Tx/4Rx: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2) Note 3: 400MHz bandwidth |  |  |  |  |  |  |  |  |  |  |

Table B.3.2.2-2. FR2, DL, InH, CG 30Mbps, 60FPS, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 15 | 9.91 | 9 | 95.37% | Note 1 |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 15 | 10.23 | 10 | 91.11% | Note 1, 3 |
| Source 20 | R1-2111351 | DDDSU | SU-MIMO | reciprocity-based precoding |  | 15 | 9.9 | 9 | 93% | Note 2, 4 |
| Source 15 | R1-2112572 | DDDSU | SU-MIMO |  | random | 15 | 11.45 | 11 | 99% | Note 1 |
| Source 7 | R1-2112551 | DDDSU | SU-MIMO | codebook-based Type 1 | random | 15 | 6.9 | 6 |  | Note 1 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 15 | 7.5 | 7 | 94% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 15 | 36 | 36 | 90% | Note 1, 5 |
| Source 14 | R1-2112296 | DDDSU | SU-MIMO | codebook-based Type 2 | random | 15 | 11 | 11 | 90.46% | Note 2 |
| Note 1: UE antenna configuraiton: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) Note 2: UE antenna configuraiton: 4Tx/4Rx: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2) Note 3: Delay aware (DA) scheduler Note 4: 64QAM Note 5: 400MHz bandwidth |  |  |  |  |  |  |  |  |  |  |

  - FR2 UL

    - DU Scenario

      - VR/CG (Pose/control-stream)

Table B.4.1.1-1. FR2, UL, DU, VR/CG 0.2Mbps, 250FPS, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 20 | 20 | 96.51% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 7.5 | 7 | 92% | Note 1, 3, 5 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 8.5 | 8 | 92% | Note 1, 2, 3, 5 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 15 | 15 | 90% | Note 1, 3, 6 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 18.5 | 18 | 91% | Note 1, 4, 5 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 26.5 | 26 | 92% | Note 1, 4, 6 |
| Source 16 | R1-2112720 | DDDUU | SU-MIMO |  | random | 10 | 18.5 | 18 | 93% | Note 1, 3, 5 |
| Source 14 | R1-2112296 | DDDSU | SU-MIMO | codebook-based Type 2 | random | 10 | >30 | >30 | 99% | Note 7 |
| Note 1: UE antenna configuraiton: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) Note 2: 400MHz bandwidth Note 3: Regular slot Note 4: with mini-slot (gNB time multiplexes multiple users within a slot by allocating 7 symbols to each UE) Note 5: Full antenna (gNB uses all its N antennas and system bandwidth for receiving pose updates from a given user in the TDM) Note 6: with combination of FDM/SDM and mini-slot (7 symbols to each UE) Note 7: UE antenna configuraiton: 4Tx/4Rx: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2) |  |  |  |  |  |  |  |  |  |  |

      - AR (1 stream: Scene/video/data/voice-stream)

Table B.4.1.2-1. FR2, UL, DU, AR (1 stream: Scene/video/data/voice-stream) 10Mbps, 60FPS, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 30 | 8.3 | 8 | 92.66% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 30 | 5 | 5 | 90% | Note 1 |
| Source 16 | R1-2112720 | DDDUU | SU-MIMO |  | random | 30 | 9 | 9 | 90% | Note 1 |
| Source 14 | R1-2112296 | DDDSU | SU-MIMO | codebook-based Type 2 | random | 30 | 1.29 | 1 | 90% | Note 2 |
| Note 1: UE antenna configuraiton: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) Note 2: UE antenna configuraiton: 4Tx/4Rx: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2) |  |  |  |  |  |  |  |  |  |  |

Table B.4.1.2-2. FR2, UL, DU, AR (1 stream: Scene/video/data/voice-stream) 20Mbps, 60FPS, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 16 | R1-2112720 | DDDUU | SU-MIMO |  | random | 60 | 5 | 5 | 90% | Note 1 |
| Source 16 | R1-2112720 | DDDUU | SU-MIMO |  | random | 15 | 3.5 | 3 | >90% | Note 1 |
| Source 16 | R1-2112720 | DDDUU | SU-MIMO |  | random | 30 | 5 | 5 | 90% | Note 1 |
| Note 1: UE antenna configuraiton: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) |  |  |  |  |  |  |  |  |  |  |

      - AR (2 streams: Pose/control-stream + scene/video/data/voice-stream)

Table B.4.1.3-1. FR2, UL, DU, AR (2 streams: Pose/control-stream + scene/video/data/voice-stream) 10.2Mbps, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | [Pose_PDB, Video_PDB] (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 16 | R1-2112720 | DDDUU | SU-MIMO |  | random | [10,30] | 4.5 | 4 | 94% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | [10,30] | 1.5 | 1 | 94% | Note 1 |
| Source 16 | R1-2112720 | DDDUU | SU-MIMO |  | random | [10,30] | 7 | 7 | 90% | Note 1, 2 |
| Note 1: UE antenna configuraiton: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) Note 2: 400MHz bandwidth |  |  |  |  |  |  |  |  |  |  |

Table B.4.1.3-2. FR2, UL, DU, AR (2 streams: Pose/control-stream + scene/video/data/voice-stream) 20.2Mbps, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | [Pose_PDB, Video_PDB] (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 16 | R1-2112720 | DDDUU | SU-MIMO |  | random | [10,30] | 2 | 2 | 90.00% | Note 1 |
| Note 1: UE antenna configuraiton: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) |  |  |  |  |  |  |  |  |  |  |

    - InH Scenario

      - VR/CG (Pose/control-stream)

Table B.4.2.1-1. FR2, UL, InH, VR/CG 0.2Mbps, 250FPS, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 10 | 20 | 20 | 97.69% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 7 | 7 | 90% | Note 1, 3, 5 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 7 | 7 | 90% | Note 1, 2, 3, 5 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 11.5 | 11 | 94% | Note 1, 3, 6 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 20 | 20 | 90% | Note 1, 4, 5 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 10 | 26 | 26 | 90% | Note 1, 4, 6 |
| Source 16 | R1-2112720 | DDDUU | SU-MIMO |  | random | 10 | 19 | 19 | 90% | Note 1, 3, 5 |
| Source 14 | R1-2112296 | DDDSU | SU-MIMO | codebook-based Type 2 | random | 10 | 12.09 | 12 | 90.28% | Note 7 |
| Note 1: UE antenna configuraiton: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) Note 2: 400MHz bandwidth Note 3: Regular slot Note 4: with mini-slot (gNB time multiplexes multiple users within a slot by allocating 7 symbols to each UE) Note 5: Full antenna (gNB uses all its N antennas and system bandwidth for receiving pose updates from a given user in the TDM) Note 6: with combination of FDM/SDM and mini-slot (7 symbols to each UE) Note 7: UE antenna configuraiton: 4Tx/4Rx: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2) |  |  |  |  |  |  |  |  |  |  |

      - AR (1 stream: Scene/video/data/voice-stream)

Table B.4.2.2-1. FR2, UL, InH, AR (1 stream: Scene/video/data/voice-stream), 10Mbps, 60FPS, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | R1-2111046 | DDDSU | SU-MIMO | reciprocity-based precoding | random | 30 | 8.59 | 8 | 95.14% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | 30 | 5 | 5 | 90% | Note 1 |
| Source 16 | R1-2112720 | DDDUU | SU-MIMO |  | random | 30 | 10 | 10 | 90% | Note 1 |
| Source 14 | R1-2112296 | DDDSU | SU-MIMO | codebook-based Type 2 | random | 30 | 1 | 1 | 90% | Note 2 |
| Note 1: UE antenna configuraiton: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) Note 2: UE antenna configuraiton: 4Tx/4Rx: (M, N, P, Mg, Ng; Mp, Np) = (2,4,2,1,2;1,2) |  |  |  |  |  |  |  |  |  |  |

Table B.4.2.2-2. FR2, UL, InH, AR (1 stream: Scene/video/data/voice-stream), 20Mbps, 60FPS, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | PDB (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 16 | R1-2112720 | DDDUU | SU-MIMO |  | random | 60 | 6 | 6 | 90% | Note 1 |
| Source 16 | R1-2112720 | DDDUU | SU-MIMO |  | random | 15 | 5 | 5 | 90% | Note 1 |
| Source 16 | R1-2112720 | DDDUU | SU-MIMO |  | random | 30 | 6 | 6 | 90% | Note 1 |
| Note 1: UE antenna configuraiton: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) |  |  |  |  |  |  |  |  |  |  |

      - AR (2 streams: Pose/control-stream + scene/video/data/voice-stream)

Table B.4.2.3-1. FR2, UL, InH, AR (2 streams: Pose/control-stream + scene/video/data/voice-stream), 10.2Mbps, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | [Pose_PDB, Video_PDB] (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 16 | R1-2112720 | DDDUU | SU-MIMO |  | random | [10,30] | 5 | 5 | 90% | Note 1 |
| Source 16 | R1-2112720 | DDDSU | SU-MIMO |  | random | [10,30] | 2.5 | 2 | 93% | Note 1 |
| Source 16 | R1-2112720 | DDDUU | SU-MIMO |  | random | [10,30] | 7.5 | 7 | 94% | Note 1, 4 |
| Source 16 | R1-2112720 | DDDUU | SU-MIMO |  | random | [10,30] | 6.5 | 6 | 95% | Note 1, 3 |
| Note 1: UE antenna configuraiton: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) Note 2: 400MHz bandwidth Note 3: Delay aware (DA) scheduler |  |  |  |  |  |  |  |  |  |  |

Table B.4.2.3-2. FR2, UL, InH, AR (2 streams: Pose/control-stream + scene/video/data/voice-stream), 20.2Mbps, SU-MIMO, 100MHz bandwidth

| Source | Tdoc Source | TDD format | SU/MU-MIMO | Transmission scheme | Traffic arrival offset among different UEs | [Pose_PDB, Video_PDB] (ms) | Capacity (UEs/cell) | C1=floor (Capacity) | % of satisfied UEs when #UEs/cell =C1 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 16 | R1-2112720 | DDDUU | SU-MIMO |  | random | [10,30] | 3.5 | 3 | 93% | Note 1 |
| Note 1: UE antenna configuraiton: (M, N, P) = (1, 4, 2), 3 panels (left, right, top) |  |  |  |  |  |  |  |  |  |  |

# Annex <C> Source Specific Power Performance Evaluation Results

Table 223 Power consumption results of CG (30Mbps) application in FR1 DL VR/CG scenario

| Source | Power Saving scheme | avg # UEs/ cell = N1 | C1=floor(Capacity) | Average PS gain (%) | % of satisfied Ues w/ PS when #UEs/cell = N1 | % of satisfied Ues w/o PS when #UEs/cell = N1 |
| --- | --- | --- | --- | --- | --- | --- |
| X | - |  |  |  |  |  |
| Y |  |  |  |  |  |  |

Note 1

# Annex <D> Source Specific Mobility Performance Evaluation Results

## D.1 Consecutive XR packets lost due to a HO event, N

Table B.1-1. FPS=30, PDB=10ms

| Company | Tdoc | F (fps) | PDB (ms) | Handover type | Handover case | Y (ms) | N (packets) | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 20 | R1-2111351 | 30 | 10 | DAPS | FR1-to-FR1 | 0 | 0 | Note 1 |
| Source 20 | R1-2111351 | 30 | 10 | DAPS | FR1-to-FR1 | 2 | 0.06 | Note 1 |
| Source 20 | R1-2111351 | 30 | 10 | DAPS | FR1-to-FR1 | 4 | 0.12 | Note 1 |
| Source 20 | R1-2111351 | 30 | 10 | DAPS | FR1-to-FR1 | 6 | 0.18 | Note 1 |
| Source 20 | R1-2111351 | 30 | 10 | DAPS | FR1-to-FR1 | 8 | 0.24 | Note 1 |
| Source 20 | R1-2111351 | 30 | 10 | DAPS | FR1-to-FR1 | 10 | 0.3 | Note 1 |
| Note 1: N = Y* F / 1000 + δ, Y < PDB, where δ = 0 |  |  |  |  |  |  |  |  |

Table B.1-2. FPS=60, PDB=10ms

| Company | Tdoc | F (fps) | PDB (ms) | Handover type | Handover case | Y (ms) | N (packets) | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | R1-2111046 | 60 | 10 | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 2.52 |  |
| Source 18 | R1-2111046 | 60 | 10 | Typical HO | FR1-to-FR1 | 62 | 3.12 |  |
| Source 18 | R1-2111046 | 60 | 10 | Typical HO | FR1-to-FR1 | 82 | 4.32 |  |
| Source 9 | R1-2110811 | 60 | 10 |  |  | 100 | 5.4 |  |
| Source 9 | R1-2110811 | 60 | 10 |  |  | 80 | 4.2 |  |
| Source 9 | R1-2110811 | 60 | 10 |  |  | 70 | 3.6 |  |
| Source 9 | R1-2110811 | 60 | 10 |  |  | 60 | 3 |  |
| Source 9 | R1-2110811 | 60 | 10 |  |  | 40 | 1.8 |  |
| Source 9 | R1-2110811 | 60 | 10 |  |  | 20 | 0.6 |  |
| Source 9 | R1-2110811 | 60 | 10 |  |  | <= 10 | 0 | Note 5 |
| Source 20 | R1-2111351 | 60 | 10 | DAPS | FR1-to-FR1 | 0 | 0 | Note 4 |
| Source 20 | R1-2111351 | 60 | 10 | DAPS | FR1-to-FR1 | 2 | 0.12 | Note 4 |
| Source 20 | R1-2111351 | 60 | 10 | DAPS | FR1-to-FR1 | 4 | 0.24 | Note 4 |
| Source 20 | R1-2111351 | 60 | 10 | DAPS | FR1-to-FR1 | 6 | 0.36 | Note 4 |
| Source 20 | R1-2111351 | 60 | 10 | DAPS | FR1-to-FR1 | 8 | 0.48 | Note 4 |
| Source 20 | R1-2111351 | 60 | 10 | DAPS | FR1-to-FR1 | 10 | 0.6 | Note 4 |
| Source 3 | R1-2111234 | 60 | 10 | Typical HO | FR1-to-FR1 | 66.5 | 3.39 |  |
| Source 3 | R1-2111234 | 60 | 10 | Typical HO | FR2-to-FR1 | 86.5 | 4.59 |  |
| Source 3 | R1-2111234 | 60 | 10 | Typical HO | FR1-to-FR2 | 79.4 | 4.164 |  |
| Source 3 | R1-2111234 | 60 | 10 | Typical HO | FR2-to-FR2 | 59.4 | 2.964 |  |
| Source 3 | R1-2111234 | 60 | 10 | Conditional HO | FR1-to-FR1 | 56.5 | 2.79 |  |
| Source 3 | R1-2111234 | 60 | 10 | Conditional HO | FR2-to-FR1 | 76.5 | 3.99 |  |
| Source 3 | R1-2111234 | 60 | 10 | Conditional HO | FR1-to-FR2 | 69.4 | 3.564 |  |
| Source 3 | R1-2111234 | 60 | 10 | Conditional HO | FR2-to-FR2 | 49.4 | 2.364 |  |
| Source 17 | R1-2111349 | 60 | 10 | Typical HO | FR1-to-FR1, FR2-to-FR2 | 47 | 2 | Note 1 |
| Source 17 | R1-2111349 | 60 | 10 | Typical HO | FR1-to-FR1, FR2-to-FR2 | 192 | 11 | Note 1 |
| Source 17 | R1-2111349 | 60 | 10 | Typical HO | FR1-to-FR1, FR2-to-FR2 | 352 | 21 | Note 1 |
| Source 17 | R1-2111349 | 60 | 10 | Typical HO | FR1-to-FR2, FR2-to-FR1 | 67 | 3 | Note 1 |
| Source 17 | R1-2111349 | 60 | 10 | Typical HO | FR1-to-FR2, FR2-to-FR1 | 212 | 12 | Note 1 |
| Source 17 | R1-2111349 | 60 | 10 | Typical HO | FR1-to-FR2, FR2-to-FR1 | 372 | 22 | Note 1 |
| Source 17 | R1-2111349 | 60 | 10 | Typical HO | FR1-to-FR1 | 52 | 3 | Note 2 |
| Source 17 | R1-2111349 | 60 | 10 | Typical HO | FR1-to-FR1 | 272 | 16 | Note 2 |
| Source 17 | R1-2111349 | 60 | 10 | Typical HO | FR1-to-FR1 | 512 | 30 | Note 2 |
| Source 17 | R1-2111349 | 60 | 10 | Typical HO | FR2-to-FR2 | 87 | 5 | Note 2 |
| Source 17 | R1-2111349 | 60 | 10 | Typical HO | FR2-to-FR2 | 832 | 49 | Note 2 |
| Source 17 | R1-2111349 | 60 | 10 | Typical HO | FR2-to-FR2 | 1632 | 97 | Note 2 |
| Source 17 | R1-2111349 | 60 | 10 | Typical HO | FR1-to-FR1 | 62 | 3 | Note 3 |
| Source 17 | R1-2111349 | 60 | 10 | Typical HO | FR1-to-FR1 | 432 | 25 | Note 3 |
| Source 17 | R1-2111349 | 60 | 10 | Typical HO | FR1-to-FR1 | 832 | 49 | Note 3 |
| Source 17 | R1-2111349 | 60 | 10 | Typical HO | FR2-to-FR1 | 82 | 4 | Note 3 |
| Source 17 | R1-2111349 | 60 | 10 | Typical HO | FR2-to-FR1 | 452 | 27 | Note 3 |
| Source 17 | R1-2111349 | 60 | 10 | Typical HO | FR2-to-FR1 | 852 | 51 | Note 3 |
| Source 17 | R1-2111349 | 60 | 10 | Typical HO | FR2-to-FR2 | 167 | 9 | Note 3 |
| Source 17 | R1-2111349 | 60 | 10 | Typical HO | FR2-to-FR2 | 2112 | 126 | Note 3 |
| Source 17 | R1-2111349 | 60 | 10 | Typical HO | FR2-to-FR2 | 4192 | 251 | Note 3 |
| Source 17 | R1-2111349 | 60 | 10 | Typical HO | FR1-to-FR2 | 187 | 11 | Note 3 |
| Source 17 | R1-2111349 | 60 | 10 | Typical HO | FR1-to-FR2 | 2132 | 127 | Note 3 |
| Source 17 | R1-2111349 | 60 | 10 | Typical HO | FR1-to-FR2 | 4212 | 252 | Note 3 |
| Source 10 | R1-2111524 | 60 | 10 | Typical HO | FR1-to-FR1 | 47 | 2.2 | Note 2 |
| Source 10 | R1-2111524 | 60 | 10 | Typical HO | FR1-to-FR1 | 142 | 7.9 | Note 2 |
| Source 15 | R1-2111828 | 60 | 10 | DAPS |  | 2 | 0.00 | Note 5 |
| Source 15 | R1-2111828 | 60 | 10 | Typical HO, Conditional HO |  | 42 | 1.93 |  |
| Source 15 | R1-2111828 | 60 | 10 | Typical HO, Conditional HO |  | 62 | 3.13 |  |
| Source 15 | R1-2111828 | 60 | 10 |  |  | 10 | 0.00 |  |
| Source 15 | R1-2111828 | 60 | 10 |  |  | 20 | 0.60 |  |
| Source 15 | R1-2111828 | 60 | 10 |  |  | 40 | 1.81 |  |
| Source 15 | R1-2111828 | 60 | 10 |  |  | 60 | 3.01 |  |
| Source 15 | R1-2111828 | 60 | 10 |  |  | 80 | 4.22 |  |
| Source 15 | R1-2111828 | 60 | 10 |  |  | 100 | 5.42 |  |
| Source 7 | R1-2112160 | 60 | 10 | DAPS |  | 2 | 0 | Note 5 |
| Source 7 | R1-2112160 | 60 | 10 |  |  | 20 | 1 |  |
| Source 7 | R1-2112160 | 60 | 10 | Typical HO, Conditional HO |  | 40 | 2 |  |
| Source 7 | R1-2112160 | 60 | 10 |  |  | 60 | 3 |  |
| Source 7 | R1-2112160 | 60 | 10 |  |  | 80 | 5 |  |
| Source 7 | R1-2112160 | 60 | 10 |  |  | 100 | 6 |  |
| Source 14 | R1-2112296 | 60 | 10 |  |  | 60 | 3.01 |  |
| Source 13 | R1-2112069 | 60 | 10 |  |  | 40 | 1.8 |  |
| Source 13 | R1-2112069 | 60 | 10 |  |  | 42 | 1.9 |  |
| Source 13 | R1-2112069 | 60 | 10 |  |  | 44 | 2.0 |  |
| Source 13 | R1-2112069 | 60 | 10 |  |  | 46 | 2.2 |  |
| Source 13 | R1-2112069 | 60 | 10 |  |  | 48 | 2.3 |  |
| Source 13 | R1-2112069 | 60 | 10 |  |  | 50 | 2.4 |  |
| Source 13 | R1-2112069 | 60 | 10 |  |  | 52 | 2.5 |  |
| Source 13 | R1-2112069 | 60 | 10 |  |  | 54 | 2.6 |  |
| Source 13 | R1-2112069 | 60 | 10 |  |  | 56 | 2.8 |  |
| Source 13 | R1-2112069 | 60 | 10 |  |  | 58 | 2.9 |  |
| Source 13 | R1-2112069 | 60 | 10 |  |  | 60 | 3.0 |  |
| Source 13 | R1-2112069 | 60 | 10 |  |  | 62 | 3.1 |  |
| Source 13 | R1-2112069 | 60 | 10 |  |  | 64 | 3.2 |  |
| Source 13 | R1-2112069 | 60 | 10 |  |  | 66 | 3.4 |  |
| Source 13 | R1-2112069 | 60 | 10 |  |  | 68 | 3.5 |  |
| Source 13 | R1-2112069 | 60 | 10 |  |  | 70 | 3.6 |  |
| Source 13 | R1-2112069 | 60 | 10 |  |  | 72 | 3.7 |  |
| Source 13 | R1-2112069 | 60 | 10 |  |  | 74 | 3.8 |  |
| Source 13 | R1-2112069 | 60 | 10 |  |  | 76 | 4.0 |  |
| Source 13 | R1-2112069 | 60 | 10 |  |  | 78 | 4.1 |  |
| Source 13 | R1-2112069 | 60 | 10 |  |  | 80 | 4.2 |  |
| Note 1: the target cell is known Note 2: target cell is an unknown intra-frequency cell Note 3: target cell is an unknown inter-frequency cell Note 4: N = Y* F / 1000 + δ, Y < PDB, where δ = 0 Note 5: N = 0, Y < PDB |  |  |  |  |  |  |  |  |

Table B.1-3. FPS=60, PDB=15ms

| Company | Tdoc | F (fps) | PDB (ms) | Handover type | Handover case | Y (ms) | N (packets) | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | R1-2111046 | 60 | 15 | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 2.22 |  |
| Source 18 | R1-2111046 | 60 | 15 | Typical HO | FR1-to-FR1 | 62 | 2.82 |  |
| Source 18 | R1-2111046 | 60 | 15 | Typical HO | FR1-to-FR1 | 82 | 4.02 |  |
| Source 3 | R1-2111234 | 60 | 15 | Typical HO | FR1-to-FR1 | 66.5 | 3.09 |  |
| Source 3 | R1-2111234 | 60 | 15 | Typical HO | FR2-to-FR1 | 86.5 | 4.29 |  |
| Source 3 | R1-2111234 | 60 | 15 | Typical HO | FR1-to-FR2 | 79.4 | 3.864 |  |
| Source 3 | R1-2111234 | 60 | 15 | Typical HO | FR2-to-FR2 | 59.4 | 2.664 |  |
| Source 3 | R1-2111234 | 60 | 15 | Conditional HO | FR1-to-FR1 | 56.5 | 2.49 |  |
| Source 3 | R1-2111234 | 60 | 15 | Conditional HO | FR2-to-FR1 | 76.5 | 3.69 |  |
| Source 3 | R1-2111234 | 60 | 15 | Conditional HO | FR1-to-FR2 | 69.4 | 3.264 |  |
| Source 3 | R1-2111234 | 60 | 15 | Conditional HO | FR2-to-FR2 | 49.4 | 2.064 |  |
| Source 6 | R1-2111632 | 60 | 15 |  |  | 48 | 1.98 |  |
| Source 15 | R1-2111828 | 60 | 15 | DAPS |  | 2 | 0.00 | Note 1 |
| Source 15 | R1-2111828 | 60 | 15 | Typical HO, Conditional HO |  | 42 | 1.63 |  |
| Source 15 | R1-2111828 | 60 | 15 | Typical HO, Conditional HO |  | 62 | 2.83 |  |
| Source 15 | R1-2111828 | 60 | 15 |  |  | 10 | 0.00 | Note 1 |
| Source 15 | R1-2111828 | 60 | 15 |  |  | 20 | 0.30 |  |
| Source 15 | R1-2111828 | 60 | 15 |  |  | 40 | 1.51 |  |
| Source 15 | R1-2111828 | 60 | 15 |  |  | 60 | 2.71 |  |
| Source 15 | R1-2111828 | 60 | 15 |  |  | 80 | 3.92 |  |
| Source 15 | R1-2111828 | 60 | 15 |  |  | 100 | 5.12 |  |
| Source 7 | R1-2112160 | 60 | 15 | DAPS |  | 2 | 0 | Note 1 |
| Source 7 | R1-2112160 | 60 | 15 |  |  | 20 | 1 |  |
| Source 7 | R1-2112160 | 60 | 15 | Typical HO, Conditional HO |  | 40 | 2 |  |
| Source 7 | R1-2112160 | 60 | 15 |  |  | 60 | 3 |  |
| Source 7 | R1-2112160 | 60 | 15 |  |  | 80 | 4 |  |
| Source 7 | R1-2112160 | 60 | 15 |  |  | 100 | 6 |  |
| Source 14 | R1-2112296 | 60 | 15 |  |  | 60 | 2.71 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 40 | 1.5 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 42 | 1.6 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 44 | 1.7 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 46 | 1.9 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 48 | 2.0 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 50 | 2.1 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 52 | 2.2 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 54 | 2.3 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 56 | 2.5 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 58 | 2.6 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 60 | 2.7 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 62 | 2.8 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 64 | 2.9 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 66 | 3.1 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 68 | 3.2 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 70 | 3.3 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 72 | 3.4 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 74 | 3.5 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 76 | 3.7 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 78 | 3.8 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 80 | 3.9 |  |
| Note 1: N = 0, Y < PDB |  |  |  |  |  |  |  |  |

Table B.1-4. FPS=60, PDB=30ms

| Company | Tdoc | F (fps) | PDB (ms) | Handover type | Handover case | Y (ms) | N (packets) | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | R1-2111046 | 60 | 30 | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 1.32 |  |
| Source 18 | R1-2111046 | 60 | 30 | Typical HO | FR1-to-FR1 | 62 | 1.92 |  |
| Source 18 | R1-2111046 | 60 | 30 | Typical HO | FR1-to-FR1 | 82 | 3.12 |  |
| Source 15 | R1-2111828 | 60 | 30 | DAPS |  | 2 | 0.00 | Note 1 |
| Source 15 | R1-2111828 | 60 | 30 | Typical HO, Conditional HO |  | 42 | 0.72 |  |
| Source 15 | R1-2111828 | 60 | 30 | Typical HO, Conditional HO |  | 62 | 1.93 |  |
| Source 15 | R1-2111828 | 60 | 30 |  |  | 10 | 0.00 | Note 1 |
| Source 15 | R1-2111828 | 60 | 30 |  |  | 20 | 0.00 | Note 1 |
| Source 15 | R1-2111828 | 60 | 30 |  |  | 40 | 0.60 |  |
| Source 15 | R1-2111828 | 60 | 30 |  |  | 60 | 1.81 |  |
| Source 15 | R1-2111828 | 60 | 30 |  |  | 80 | 3.01 |  |
| Source 15 | R1-2111828 | 60 | 30 |  |  | 100 | 4.22 |  |
| Source 7 | R1-2112160 | 60 | 30 | DAPS |  | 2 | 0 | Note 1 |
| Source 7 | R1-2112160 | 60 | 30 |  |  | 20 | 0 | Note 1 |
| Source 7 | R1-2112160 | 60 | 30 | Typical HO, Conditional HO |  | 40 | 1 |  |
| Source 7 | R1-2112160 | 60 | 30 |  |  | 60 | 2 |  |
| Source 7 | R1-2112160 | 60 | 30 |  |  | 80 | 3 |  |
| Source 7 | R1-2112160 | 60 | 30 |  |  | 100 | 5 |  |
| Source 14 | R1-2112296 | 60 | 30 |  |  | 60 | 1.81 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 40 | 0.6 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 42 | 0.7 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 44 | 0.8 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 46 | 1.0 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 48 | 1.1 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 50 | 1.2 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 52 | 1.3 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 54 | 1.4 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 56 | 1.6 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 58 | 1.7 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 60 | 1.8 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 62 | 1.9 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 64 | 2.0 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 66 | 2.2 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 68 | 2.3 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 70 | 2.4 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 72 | 2.5 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 74 | 2.6 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 76 | 2.8 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 78 | 2.9 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 80 | 3.0 |  |
| Note 1: N = 0, Y < PDB |  |  |  |  |  |  |  |  |

Table B.1-5. FPS=90, PDB=10/15/30ms

| Company | Tdoc | F (fps) | PDB (ms) | Handover type | Handover case | Y (ms) | N (packets) | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | R1-2111046 | 90 | 10 | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 3.78 |  |
| Source 18 | R1-2111046 | 90 | 10 | Typical HO | FR1-to-FR1 | 62 | 4.68 |  |
| Source 18 | R1-2111046 | 90 | 10 | Typical HO | FR1-to-FR1 | 82 | 6.48 |  |
| Source 18 | R1-2111046 | 90 | 15 | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 3.33 |  |
| Source 18 | R1-2111046 | 90 | 15 | Typical HO | FR1-to-FR1 | 62 | 4.23 |  |
| Source 18 | R1-2111046 | 90 | 15 | Typical HO | FR1-to-FR1 | 82 | 6.03 |  |
| Source 18 | R1-2111046 | 90 | 30 | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 1.98 |  |
| Source 18 | R1-2111046 | 90 | 30 | Typical HO | FR1-to-FR1 | 62 | 2.88 |  |
| Source 18 | R1-2111046 | 90 | 30 | Typical HO | FR1-to-FR1 | 82 | 4.68 |  |

Table B.1-6. FPS=120, PDB=10/15/30ms

| Company | Tdoc | F (fps) | PDB (ms) | Handover type | Handover case | Y (ms) | N (packets) | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | R1-2111046 | 120 | 10 | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 5.04 |  |
| Source 18 | R1-2111046 | 120 | 10 | Typical HO | FR1-to-FR1 | 62 | 6.24 |  |
| Source 18 | R1-2111046 | 120 | 10 | Typical HO | FR1-to-FR1 | 82 | 8.64 |  |
| Source 18 | R1-2111046 | 120 | 15 | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 4.44 |  |
| Source 18 | R1-2111046 | 120 | 15 | Typical HO | FR1-to-FR1 | 62 | 5.64 |  |
| Source 18 | R1-2111046 | 120 | 15 | Typical HO | FR1-to-FR1 | 82 | 8.04 |  |
| Source 18 | R1-2111046 | 120 | 30 | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 2.64 |  |
| Source 18 | R1-2111046 | 120 | 30 | Typical HO | FR1-to-FR1 | 62 | 3.84 |  |
| Source 18 | R1-2111046 | 120 | 30 | Typical HO | FR1-to-FR1 | 82 | 6.24 |  |
| Source 20 | R1-2111351 | 120 | 10 | DAPS | FR1-to-FR1 | 0 | 0 | Note 1 |
| Source 20 | R1-2111351 | 120 | 10 | DAPS | FR1-to-FR1 | 2 | 0.24 | Note 1 |
| Source 20 | R1-2111351 | 120 | 10 | DAPS | FR1-to-FR1 | 4 | 0.48 | Note 1 |
| Source 20 | R1-2111351 | 120 | 10 | DAPS | FR1-to-FR1 | 6 | 0.72 | Note 1 |
| Source 20 | R1-2111351 | 120 | 10 | DAPS | FR1-to-FR1 | 8 | 0.96 | Note 1 |
| Source 20 | R1-2111351 | 120 | 10 | DAPS | FR1-to-FR1 | 10 | 1.2 | Note 1 |
| Note 1: N = Y* F / 1000 + δ, Y < PDB, where δ = 0 |  |  |  |  |  |  |  |  |

Table B.1-7. FPS=250, PDB=10ms

| Company | Tdoc | F (fps) | PDB (ms) | Handover type | Handover case | Y (ms) | N (packets) | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | R1-2111046 | 250 | 10 | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 10.50 |  |
| Source 18 | R1-2111046 | 250 | 10 | Typical HO | FR1-to-FR1 | 62 | 13.00 |  |
| Source 18 | R1-2111046 | 250 | 10 | Typical HO | FR1-to-FR1 | 82 | 18.00 |  |
| Source 15 | R1-2111828 | 250 | 10 | DAPS |  | 2 | 0.00 | Note 1 |
| Source 15 | R1-2111828 | 250 | 10 | Typical HO, Conditional HO |  | 42 | 8.00 |  |
| Source 15 | R1-2111828 | 250 | 10 | Typical HO, Conditional HO |  | 62 | 13.00 |  |
| Source 15 | R1-2111828 | 250 | 10 |  |  | 10 | 0.00 |  |
| Source 15 | R1-2111828 | 250 | 10 |  |  | 20 | 2.50 |  |
| Source 15 | R1-2111828 | 250 | 10 |  |  | 40 | 7.50 |  |
| Source 15 | R1-2111828 | 250 | 10 |  |  | 60 | 12.50 |  |
| Source 15 | R1-2111828 | 250 | 10 |  |  | 80 | 17.50 |  |
| Source 15 | R1-2111828 | 250 | 10 |  |  | 100 | 22.50 |  |
| Source 7 | R1-2112160 | 250 | 10 | DAPS |  | 2 | 0 | Note 1 |
| Source 7 | R1-2112160 | 250 | 10 |  |  | 20 | 3 |  |
| Source 7 | R1-2112160 | 250 | 10 | Typical HO, Conditional HO |  | 40 | 8 |  |
| Source 7 | R1-2112160 | 250 | 10 |  |  | 60 | 13 |  |
| Source 7 | R1-2112160 | 250 | 10 |  |  | 80 | 18 |  |
| Source 7 | R1-2112160 | 250 | 10 |  |  | 100 | 23 |  |
| Source 14 | R1-2112296 | 250 | 10 |  |  | 60 | 12.5 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 40 | 7.5 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 42 | 8.0 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 44 | 8.5 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 46 | 9.0 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 48 | 9.5 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 50 | 10.0 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 52 | 10.5 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 54 | 11.0 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 56 | 11.5 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 58 | 12.0 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 60 | 12.5 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 62 | 13.0 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 64 | 13.5 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 66 | 14.0 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 68 | 14.5 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 70 | 15.0 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 72 | 15.5 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 74 | 16.0 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 76 | 16.5 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 78 | 17.0 |  |
| Source 13 | R1-2112069 | 60 | 15 |  |  | 80 | 17.5 |  |
| Note 1: N = 0, Y < PDB |  |  |  |  |  |  |  |  |

## D.2 Minimum target time interval between HO events, T

Table B.2-1. X=99%, PDB=10ms

| Company | Tdoc | PDB (ms) | PE,op | X | Handover type | Handover case | Y (ms) | T (s) | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | R1-2111046 | 10 | 0.00% | 99.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 4.20 |  |
| Source 18 | R1-2111046 | 10 | 0.00% | 99.0% | Typical HO | FR1-to-FR1 | 62 | 5.20 |  |
| Source 18 | R1-2111046 | 10 | 0.00% | 99.0% | Typical HO | FR1-to-FR1 | 82 | 7.20 |  |
| Source 18 | R1-2111046 | 10 | 0.10% | 99.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 4.66 |  |
| Source 18 | R1-2111046 | 10 | 0.10% | 99.0% | Typical HO | FR1-to-FR1 | 62 | 5.77 |  |
| Source 18 | R1-2111046 | 10 | 0.10% | 99.0% | Typical HO | FR1-to-FR1 | 82 | 7.99 |  |
| Source 18 | R1-2111046 | 10 | 0.50% | 99.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 8.36 |  |
| Source 18 | R1-2111046 | 10 | 0.50% | 99.0% | Typical HO | FR1-to-FR1 | 62 | 10.35 |  |
| Source 18 | R1-2111046 | 10 | 0.50% | 99.0% | Typical HO | FR1-to-FR1 | 82 | 14.33 |  |
| Source 18 | R1-2111046 | 10 | 0.90% | 99.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 41.62 |  |
| Source 18 | R1-2111046 | 10 | 0.90% | 99.0% | Typical HO | FR1-to-FR1 | 62 | 51.53 |  |
| Source 18 | R1-2111046 | 10 | 0.90% | 99.0% | Typical HO | FR1-to-FR1 | 82 | 71.35 |  |
| Source 9 | R1-2110811 | 10 | 0.00% | 99.0% |  |  | 100 | 9.00 |  |
| Source 9 | R1-2110811 | 10 | 0.00% | 99.0% |  |  | 80 | 7.00 |  |
| Source 9 | R1-2110811 | 10 | 0.00% | 99.0% |  |  | 70 | 6.00 |  |
| Source 9 | R1-2110811 | 10 | 0.00% | 99.0% |  |  | 60 | 5.00 |  |
| Source 9 | R1-2110811 | 10 | 0.00% | 99.0% |  |  | 40 | 3.00 |  |
| Source 9 | R1-2110811 | 10 | 0.00% | 99.0% |  |  | 20 | 1.00 |  |
| Source 9 | R1-2110811 | 10 | 0.00% | 99.0% |  |  | <= 10 | 0.00 | Note 4 |
| Source 9 | R1-2110811 | 10 | 0.14% | 99.0% |  |  | 100 | 10.45 |  |
| Source 9 | R1-2110811 | 10 | 0.14% | 99.0% |  |  | 80 | 8.13 |  |
| Source 9 | R1-2110811 | 10 | 0.14% | 99.0% |  |  | 70 | 6.97 |  |
| Source 9 | R1-2110811 | 10 | 0.14% | 99.0% |  |  | 60 | 5.81 |  |
| Source 9 | R1-2110811 | 10 | 0.14% | 99.0% |  |  | 40 | 3.48 |  |
| Source 9 | R1-2110811 | 10 | 0.14% | 99.0% |  |  | 20 | 1.16 |  |
| Source 9 | R1-2110811 | 10 | 0.14% | 99.0% |  |  | <= 10 | 0.00 | Note 4 |
| Source 3 | R1-2111234 | 10 | 0.01% | 99.0% | Typical HO | FR1-to-FR1 | 66.5 | 4.697 |  |
| Source 3 | R1-2111234 | 10 | 0.01% | 99.0% | Typical HO | FR2-to-FR2 | 59.4 | 3.979 |  |
| Source 3 | R1-2111234 | 10 | 0.03% | 99.0% | Typical HO | FR1-to-FR1 | 66.5 | 4.792 |  |
| Source 3 | R1-2111234 | 10 | 0.03% | 99.0% | Typical HO | FR2-to-FR2 | 59.4 | 4.061 |  |
| Source 3 | R1-2111234 | 10 | 0.31% | 99.0% | Typical HO | FR1-to-FR1 | 66.5 | 6.718 |  |
| Source 3 | R1-2111234 | 10 | 0.31% | 99.0% | Typical HO | FR2-to-FR2 | 59.4 | 5.692 |  |
| Source 3 | R1-2111234 | 10 | 0.01% | 99.0% | Conditional HO | FR1-to-FR1 | 56.5 | 4.697 |  |
| Source 3 | R1-2111234 | 10 | 0.01% | 99.0% | Conditional HO | FR2-to-FR2 | 49.4 | 3.979 |  |
| Source 3 | R1-2111234 | 10 | 0.03% | 99.0% | Conditional HO | FR1-to-FR1 | 56.5 | 4.997 |  |
| Source 3 | R1-2111234 | 10 | 0.03% | 99.0% | Conditional HO | FR2-to-FR2 | 49.4 | 4.234 |  |
| Source 3 | R1-2111234 | 10 | 0.31% | 99.0% | Conditional HO | FR1-to-FR1 | 56.5 | 6.185 |  |
| Source 3 | R1-2111234 | 10 | 0.31% | 99.0% | Conditional HO | FR2-to-FR2 | 49.4 | 5.24 |  |
| Source 17 | R1-2111349 | 10 | 0.01% | 99.0% | Typical HO | FR1-to-FR1, FR2-to-FR2 | 47 | 4 | Note 1 |
| Source 17 | R1-2111349 | 10 | 0.01% | 99.0% | Typical HO | FR1-to-FR1, FR2-to-FR2 | 192 | 18 | Note 1 |
| Source 17 | R1-2111349 | 10 | 0.01% | 99.0% | Typical HO | FR1-to-FR1, FR2-to-FR2 | 352 | 35 | Note 1 |
| Source 17 | R1-2111349 | 10 | 0.01% | 99.0% | Typical HO | FR1-to-FR2, FR2-to-FR1 | 67 | 6 | Note 1 |
| Source 17 | R1-2111349 | 10 | 0.01% | 99.0% | Typical HO | FR1-to-FR2, FR2-to-FR1 | 212 | 20 | Note 1 |
| Source 17 | R1-2111349 | 10 | 0.01% | 99.0% | Typical HO | FR1-to-FR2, FR2-to-FR1 | 372 | 37 | Note 1 |
| Source 17 | R1-2111349 | 10 | 0.10% | 99.0% | Typical HO | FR1-to-FR1, FR2-to-FR2 | 47 | 4 | Note 2 |
| Source 17 | R1-2111349 | 10 | 0.10% | 99.0% | Typical HO | FR1-to-FR1, FR2-to-FR2 | 192 | 20 | Note 1 |
| Source 17 | R1-2111349 | 10 | 0.10% | 99.0% | Typical HO | FR1-to-FR1, FR2-to-FR2 | 352 | 38 | Note 1 |
| Source 17 | R1-2111349 | 10 | 0.10% | 99.0% | Typical HO | FR1-to-FR2, FR2-to-FR1 | 67 | 6 | Note 1 |
| Source 17 | R1-2111349 | 10 | 0.10% | 99.0% | Typical HO | FR1-to-FR2, FR2-to-FR1 | 212 | 22 | Note 1 |
| Source 17 | R1-2111349 | 10 | 0.10% | 99.0% | Typical HO | FR1-to-FR2, FR2-to-FR1 | 372 | 40 | Note 1 |
| Source 17 | R1-2111349 | 10 | 0.50% | 99.0% | Typical HO | FR1-to-FR1, FR2-to-FR2 | 47 | 7 | Note 1 |
| Source 17 | R1-2111349 | 10 | 0.50% | 99.0% | Typical HO | FR1-to-FR1, FR2-to-FR2 | 192 | 36 | Note 1 |
| Source 17 | R1-2111349 | 10 | 0.50% | 99.0% | Typical HO | FR1-to-FR1, FR2-to-FR2 | 352 | 68 | Note 1 |
| Source 17 | R1-2111349 | 10 | 0.50% | 99.0% | Typical HO | FR1-to-FR2, FR2-to-FR1 | 67 | 11 | Note 1 |
| Source 17 | R1-2111349 | 10 | 0.50% | 99.0% | Typical HO | FR1-to-FR2, FR2-to-FR1 | 212 | 40 | Note 1 |
| Source 17 | R1-2111349 | 10 | 0.50% | 99.0% | Typical HO | FR1-to-FR2, FR2-to-FR1 | 372 | 72 | Note 1 |
| Source 17 | R1-2111349 | 10 | 0.01% | 99.0% | Typical HO | FR1-to-FR1 | 52 | 4 | Note 2 |
| Source 17 | R1-2111349 | 10 | 0.01% | 99.0% | Typical HO | FR1-to-FR1 | 272 | 26 | Note 2 |
| Source 17 | R1-2111349 | 10 | 0.01% | 99.0% | Typical HO | FR1-to-FR1 | 512 | 51 | Note 2 |
| Source 17 | R1-2111349 | 10 | 0.01% | 99.0% | Typical HO | FR2-to-FR2 | 87 | 8 | Note 2 |
| Source 17 | R1-2111349 | 10 | 0.01% | 99.0% | Typical HO | FR2-to-FR2 | 832 | 83 | Note 2 |
| Source 17 | R1-2111349 | 10 | 0.01% | 99.0% | Typical HO | FR2-to-FR2 | 1632 | 164 | Note 2 |
| Source 17 | R1-2111349 | 10 | 0.10% | 99.0% | Typical HO | FR1-to-FR1 | 52 | 5 | Note 2 |
| Source 17 | R1-2111349 | 10 | 0.10% | 99.0% | Typical HO | FR1-to-FR1 | 272 | 29 | Note 2 |
| Source 17 | R1-2111349 | 10 | 0.10% | 99.0% | Typical HO | FR1-to-FR1 | 512 | 56 | Note 2 |
| Source 17 | R1-2111349 | 10 | 0.10% | 99.0% | Typical HO | FR2-to-FR2 | 87 | 9 | Note 2 |
| Source 17 | R1-2111349 | 10 | 0.10% | 99.0% | Typical HO | FR2-to-FR2 | 832 | 91 | Note 2 |
| Source 17 | R1-2111349 | 10 | 0.10% | 99.0% | Typical HO | FR2-to-FR2 | 1632 | 180 | Note 2 |
| Source 17 | R1-2111349 | 10 | 0.50% | 99.0% | Typical HO | FR1-to-FR1 | 52 | 8 | Note 2 |
| Source 17 | R1-2111349 | 10 | 0.50% | 99.0% | Typical HO | FR1-to-FR1 | 272 | 52 | Note 2 |
| Source 17 | R1-2111349 | 10 | 0.50% | 99.0% | Typical HO | FR1-to-FR1 | 512 | 100 | Note 2 |
| Source 17 | R1-2111349 | 10 | 0.50% | 99.0% | Typical HO | FR2-to-FR2 | 87 | 15 | Note 2 |
| Source 17 | R1-2111349 | 10 | 0.50% | 99.0% | Typical HO | FR2-to-FR2 | 832 | 164 | Note 2 |
| Source 17 | R1-2111349 | 10 | 0.50% | 99.0% | Typical HO | FR2-to-FR2 | 1632 | 323 | Note 2 |
| Source 17 | R1-2111349 | 10 | 0.01% | 99.0% | Typical HO | FR1-to-FR1 | 62 | 5 | Note 3 |
| Source 17 | R1-2111349 | 10 | 0.01% | 99.0% | Typical HO | FR1-to-FR1 | 432 | 43 | Note 3 |
| Source 17 | R1-2111349 | 10 | 0.01% | 99.0% | Typical HO | FR1-to-FR1 | 832 | 83 | Note 3 |
| Source 17 | R1-2111349 | 10 | 0.10% | 99.0% | Typical HO | FR1-to-FR1 | 62 | 6 | Note 3 |
| Source 17 | R1-2111349 | 10 | 0.10% | 99.0% | Typical HO | FR1-to-FR1 | 432 | 47 | Note 3 |
| Source 17 | R1-2111349 | 10 | 0.10% | 99.0% | Typical HO | FR1-to-FR1 | 832 | 91 | Note 3 |
| Source 17 | R1-2111349 | 10 | 0.50% | 99.0% | Typical HO | FR1-to-FR1 | 62 | 10 | Note 3 |
| Source 17 | R1-2111349 | 10 | 0.50% | 99.0% | Typical HO | FR1-to-FR1 | 432 | 84 | Note 3 |
| Source 17 | R1-2111349 | 10 | 0.50% | 99.0% | Typical HO | FR1-to-FR1 | 832 | 164 | Note 3 |
| Source 17 | R1-2111349 | 10 | 0.01% | 99.0% | Typical HO | FR2-to-FR1 | 82 | 7 | Note 3 |
| Source 17 | R1-2111349 | 10 | 0.01% | 99.0% | Typical HO | FR2-to-FR1 | 452 | 45 | Note 3 |
| Source 17 | R1-2111349 | 10 | 0.01% | 99.0% | Typical HO | FR2-to-FR1 | 852 | 85 | Note 3 |
| Source 17 | R1-2111349 | 10 | 0.10% | 99.0% | Typical HO | FR2-to-FR1 | 82 | 8 | Note 3 |
| Source 17 | R1-2111349 | 10 | 0.10% | 99.0% | Typical HO | FR2-to-FR1 | 452 | 49 | Note 3 |
| Source 17 | R1-2111349 | 10 | 0.10% | 99.0% | Typical HO | FR2-to-FR1 | 852 | 93 | Note 3 |
| Source 17 | R1-2111349 | 10 | 0.50% | 99.0% | Typical HO | FR2-to-FR1 | 82 | 14 | Note 3 |
| Source 17 | R1-2111349 | 10 | 0.50% | 99.0% | Typical HO | FR2-to-FR1 | 452 | 88 | Note 3 |
| Source 17 | R1-2111349 | 10 | 0.50% | 99.0% | Typical HO | FR2-to-FR1 | 852 | 168 | Note 3 |
| Source 17 | R1-2111349 | 10 | 0.01% | 99.0% | Typical HO | FR2-to-FR2 | 167 | 16 | Note 3 |
| Source 17 | R1-2111349 | 10 | 0.01% | 99.0% | Typical HO | FR2-to-FR2 | 2112 | 212 | Note 3 |
| Source 17 | R1-2111349 | 10 | 0.01% | 99.0% | Typical HO | FR2-to-FR2 | 4192 | 422 | Note 3 |
| Source 17 | R1-2111349 | 10 | 0.10% | 99.0% | Typical HO | FR2-to-FR2 | 167 | 17 | Note 3 |
| Source 17 | R1-2111349 | 10 | 0.10% | 99.0% | Typical HO | FR2-to-FR2 | 2112 | 233 | Note 3 |
| Source 17 | R1-2111349 | 10 | 0.10% | 99.0% | Typical HO | FR2-to-FR2 | 4192 | 464 | Note 3 |
| Source 17 | R1-2111349 | 10 | 0.50% | 99.0% | Typical HO | FR2-to-FR2 | 167 | 31 | Note 3 |
| Source 17 | R1-2111349 | 10 | 0.50% | 99.0% | Typical HO | FR2-to-FR2 | 2112 | 418 | Note 3 |
| Source 17 | R1-2111349 | 10 | 0.50% | 99.0% | Typical HO | FR2-to-FR2 | 4192 | 832 | Note 3 |
| Source 17 | R1-2111349 | 10 | 0.01% | 99.0% | Typical HO | FR1-to-FR2 | 187 | 18 | Note 3 |
| Source 17 | R1-2111349 | 10 | 0.01% | 99.0% | Typical HO | FR1-to-FR2 | 2132 | 214 | Note 3 |
| Source 17 | R1-2111349 | 10 | 0.01% | 99.0% | Typical HO | FR1-to-FR2 | 4212 | 424 | Note 3 |
| Source 17 | R1-2111349 | 10 | 0.10% | 99.0% | Typical HO | FR1-to-FR2 | 187 | 20 | Note 3 |
| Source 17 | R1-2111349 | 10 | 0.10% | 99.0% | Typical HO | FR1-to-FR2 | 2132 | 236 | Note 3 |
| Source 17 | R1-2111349 | 10 | 0.10% | 99.0% | Typical HO | FR1-to-FR2 | 4212 | 466 | Note 3 |
| Source 17 | R1-2111349 | 10 | 0.50% | 99.0% | Typical HO | FR1-to-FR2 | 187 | 35 | Note 3 |
| Source 17 | R1-2111349 | 10 | 0.50% | 99.0% | Typical HO | FR1-to-FR2 | 2132 | 422 | Note 3 |
| Source 17 | R1-2111349 | 10 | 0.50% | 99.0% | Typical HO | FR1-to-FR2 | 4212 | 836 | Note 3 |
| Source 15 | R1-2111828 | 10 | 0.00% | 99.0% | DAPS |  | 2 | 0 | Note 4 |
| Source 15 | R1-2111828 | 10 | 0.00% | 99.0% | Typical HO, Conditional HO |  | 42 | 3.2 |  |
| Source 15 | R1-2111828 | 10 | 0.00% | 99.0% | Typical HO, Conditional HO |  | 62 | 5.2 |  |
| Source 15 | R1-2111828 | 10 | 0.00% | 99.0% |  |  | 10 | 0 |  |
| Source 15 | R1-2111828 | 10 | 0.00% | 99.0% |  |  | 20 | 1 |  |
| Source 15 | R1-2111828 | 10 | 0.00% | 99.0% |  |  | 40 | 3 |  |
| Source 15 | R1-2111828 | 10 | 0.00% | 99.0% |  |  | 60 | 5 |  |
| Source 15 | R1-2111828 | 10 | 0.00% | 99.0% |  |  | 80 | 7 |  |
| Source 15 | R1-2111828 | 10 | 0.00% | 99.0% |  |  | 100 | 9 |  |
| Source 15 | R1-2111828 | 10 | 0.20% | 99.0% | DAPS |  | 2 | 0.00 | Note 4 |
| Source 15 | R1-2111828 | 10 | 0.20% | 99.0% | Typical HO, Conditional HO |  | 42 | 3.99 |  |
| Source 15 | R1-2111828 | 10 | 0.20% | 99.0% | Typical HO, Conditional HO |  | 62 | 6.49 |  |
| Source 15 | R1-2111828 | 10 | 0.20% | 99.0% |  |  | 10 | 0.00 |  |
| Source 15 | R1-2111828 | 10 | 0.20% | 99.0% |  |  | 20 | 1.25 |  |
| Source 15 | R1-2111828 | 10 | 0.20% | 99.0% |  |  | 40 | 3.74 |  |
| Source 15 | R1-2111828 | 10 | 0.20% | 99.0% |  |  | 60 | 6.24 |  |
| Source 15 | R1-2111828 | 10 | 0.20% | 99.0% |  |  | 80 | 8.73 |  |
| Source 15 | R1-2111828 | 10 | 0.20% | 99.0% |  |  | 100 | 11.23 |  |
| Source 15 | R1-2111828 | 10 | 0.40% | 99.0% | DAPS |  | 2 | 0.00 | Note 4 |
| Source 15 | R1-2111828 | 10 | 0.40% | 99.0% | Typical HO, Conditional HO |  | 42 | 5.31 |  |
| Source 15 | R1-2111828 | 10 | 0.40% | 99.0% | Typical HO, Conditional HO |  | 62 | 8.63 |  |
| Source 15 | R1-2111828 | 10 | 0.40% | 99.0% |  |  | 10 | 0.00 |  |
| Source 15 | R1-2111828 | 10 | 0.40% | 99.0% |  |  | 20 | 1.66 |  |
| Source 15 | R1-2111828 | 10 | 0.40% | 99.0% |  |  | 40 | 4.98 |  |
| Source 15 | R1-2111828 | 10 | 0.40% | 99.0% |  |  | 60 | 8.30 |  |
| Source 15 | R1-2111828 | 10 | 0.40% | 99.0% |  |  | 80 | 11.62 |  |
| Source 15 | R1-2111828 | 10 | 0.40% | 99.0% |  |  | 100 | 14.94 |  |
| Source 15 | R1-2111828 | 10 | 0.60% | 99.0% | DAPS |  | 2 | 0.00 | Note 4 |
| Source 15 | R1-2111828 | 10 | 0.60% | 99.0% | Typical HO, Conditional HO |  | 42 | 7.95 |  |
| Source 15 | R1-2111828 | 10 | 0.60% | 99.0% | Typical HO, Conditional HO |  | 62 | 12.92 |  |
| Source 15 | R1-2111828 | 10 | 0.60% | 99.0% |  |  | 10 | 0.00 |  |
| Source 15 | R1-2111828 | 10 | 0.60% | 99.0% |  |  | 20 | 2.48 |  |
| Source 15 | R1-2111828 | 10 | 0.60% | 99.0% |  |  | 40 | 7.45 |  |
| Source 15 | R1-2111828 | 10 | 0.60% | 99.0% |  |  | 60 | 12.43 |  |
| Source 15 | R1-2111828 | 10 | 0.60% | 99.0% |  |  | 80 | 17.40 |  |
| Source 15 | R1-2111828 | 10 | 0.60% | 99.0% |  |  | 100 | 22.36 |  |
| Source 15 | R1-2111828 | 10 | 0.80% | 99.0% | DAPS |  | 2 | 0.00 | Note 4 |
| Source 15 | R1-2111828 | 10 | 0.80% | 99.0% | Typical HO, Conditional HO |  | 42 | 15.87 |  |
| Source 15 | R1-2111828 | 10 | 0.80% | 99.0% | Typical HO, Conditional HO |  | 62 | 25.79 |  |
| Source 15 | R1-2111828 | 10 | 0.80% | 99.0% |  |  | 10 | 0.00 |  |
| Source 15 | R1-2111828 | 10 | 0.80% | 99.0% |  |  | 20 | 4.96 |  |
| Source 15 | R1-2111828 | 10 | 0.80% | 99.0% |  |  | 40 | 14.88 |  |
| Source 15 | R1-2111828 | 10 | 0.80% | 99.0% |  |  | 60 | 24.80 |  |
| Source 15 | R1-2111828 | 10 | 0.80% | 99.0% |  |  | 80 | 34.72 |  |
| Source 15 | R1-2111828 | 10 | 0.80% | 99.0% |  |  | 100 | 44.64 |  |
| Source 15 | R1-2111828 | 10 | 0.90% | 99.0% | DAPS |  | 2 | 0.00 | Note 4 |
| Source 15 | R1-2111828 | 10 | 0.90% | 99.0% | Typical HO, Conditional HO |  | 42 | 31.71 |  |
| Source 15 | R1-2111828 | 10 | 0.90% | 99.0% | Typical HO, Conditional HO |  | 62 | 51.53 |  |
| Source 15 | R1-2111828 | 10 | 0.90% | 99.0% |  |  | 10 | 0.00 |  |
| Source 15 | R1-2111828 | 10 | 0.90% | 99.0% |  |  | 20 | 9.91 |  |
| Source 15 | R1-2111828 | 10 | 0.90% | 99.0% |  |  | 40 | 29.73 |  |
| Source 15 | R1-2111828 | 10 | 0.90% | 99.0% |  |  | 60 | 49.55 |  |
| Source 15 | R1-2111828 | 10 | 0.90% | 99.0% |  |  | 80 | 69.37 |  |
| Source 15 | R1-2111828 | 10 | 0.90% | 99.0% |  |  | 100 | 89.19 |  |
| Source 7 | R1-2112160 | 10 | 0.00% | 99.0% | DAPS |  | 2 | 0.0 | Note 4 |
| Source 7 | R1-2112160 | 10 | 0.00% | 99.0% |  |  | 20 | 1.0 |  |
| Source 7 | R1-2112160 | 10 | 0.00% | 99.0% | Typical HO, Conditional HO |  | 40 | 3.0 |  |
| Source 7 | R1-2112160 | 10 | 0.00% | 99.0% |  |  | 60 | 5.0 |  |
| Source 7 | R1-2112160 | 10 | 0.00% | 99.0% |  |  | 80 | 7.0 |  |
| Source 7 | R1-2112160 | 10 | 0.00% | 99.0% |  |  | 100 | 9.0 |  |
| Source 7 | R1-2112160 | 10 | 0.20% | 99.0% | DAPS |  | 2 | 0.0 | Note 4 |
| Source 7 | R1-2112160 | 10 | 0.20% | 99.0% |  |  | 20 | 1.2 |  |
| Source 7 | R1-2112160 | 10 | 0.20% | 99.0% | Typical HO, Conditional HO |  | 40 | 3.7 |  |
| Source 7 | R1-2112160 | 10 | 0.20% | 99.0% |  |  | 60 | 6.2 |  |
| Source 7 | R1-2112160 | 10 | 0.20% | 99.0% |  |  | 80 | 8.7 |  |
| Source 7 | R1-2112160 | 10 | 0.20% | 99.0% |  |  | 100 | 11.2 |  |
| Source 7 | R1-2112160 | 10 | 0.40% | 99.0% | DAPS |  | 2 | 0.0 | Note 4 |
| Source 7 | R1-2112160 | 10 | 0.40% | 99.0% |  |  | 20 | 1.7 |  |
| Source 7 | R1-2112160 | 10 | 0.40% | 99.0% | Typical HO, Conditional HO |  | 40 | 5.0 |  |
| Source 7 | R1-2112160 | 10 | 0.40% | 99.0% |  |  | 60 | 8.3 |  |
| Source 7 | R1-2112160 | 10 | 0.40% | 99.0% |  |  | 80 | 11.6 |  |
| Source 7 | R1-2112160 | 10 | 0.40% | 99.0% |  |  | 100 | 14.9 |  |
| Source 7 | R1-2112160 | 10 | 0.60% | 99.0% | DAPS |  | 2 | 0.0 | Note 4 |
| Source 7 | R1-2112160 | 10 | 0.60% | 99.0% |  |  | 20 | 2.5 |  |
| Source 7 | R1-2112160 | 10 | 0.60% | 99.0% | Typical HO, Conditional HO |  | 40 | 7.5 |  |
| Source 7 | R1-2112160 | 10 | 0.60% | 99.0% |  |  | 60 | 12.4 |  |
| Source 7 | R1-2112160 | 10 | 0.60% | 99.0% |  |  | 80 | 17.4 |  |
| Source 7 | R1-2112160 | 10 | 0.60% | 99.0% |  |  | 100 | 22.4 |  |
| Source 7 | R1-2112160 | 10 | 0.80% | 99.0% | DAPS |  | 2 | 0.0 | Note 4 |
| Source 7 | R1-2112160 | 10 | 0.80% | 99.0% |  |  | 20 | 5.0 |  |
| Source 7 | R1-2112160 | 10 | 0.80% | 99.0% | Typical HO, Conditional HO |  | 40 | 14.9 |  |
| Source 7 | R1-2112160 | 10 | 0.80% | 99.0% |  |  | 60 | 24.8 |  |
| Source 7 | R1-2112160 | 10 | 0.80% | 99.0% |  |  | 80 | 34.7 |  |
| Source 7 | R1-2112160 | 10 | 0.80% | 99.0% |  |  | 100 | 44.6 |  |
| Source 7 | R1-2112160 | 10 | 0.90% | 99.0% | DAPS |  | 2 | 0.0 | Note 4 |
| Source 7 | R1-2112160 | 10 | 0.90% | 99.0% |  |  | 20 | 9.9 |  |
| Source 7 | R1-2112160 | 10 | 0.90% | 99.0% | Typical HO, Conditional HO |  | 40 | 29.7 |  |
| Source 7 | R1-2112160 | 10 | 0.90% | 99.0% |  |  | 60 | 49.5 |  |
| Source 7 | R1-2112160 | 10 | 0.90% | 99.0% |  |  | 80 | 69.4 |  |
| Source 7 | R1-2112160 | 10 | 0.90% | 99.0% |  |  | 100 | 89.2 |  |
| Source 14 | R1-2112296 | 10 | 0.00% | 99.0% |  |  | 60 | 5 |  |
| Source 14 | R1-2112296 | 10 | 0.5% | 99.0% |  |  | 60 | 9.95 |  |
| Source 14 | R1-2112296 | 10 | 0.9% | 99.0% |  |  | 60 | 49.55 |  |
| Source 10 | R1-2111524 | 10 | 0.01% | 99.0% | Typical HO | FR1-FR1 | 47 | 3.73 | Note 2 |
| Source 10 | R1-2111525 | 10 | 0.10% | 99.0% | Typical HO | FR1-FR1 | 47 | 4.107 | Note 2 |
| Source 10 | R1-2111526 | 10 | 0.20% | 99.0% | Typical HO | FR1-FR1 | 47 | 4.616 | Note 2 |
| Source 10 | R1-2111527 | 10 | 0.30% | 99.0% | Typical HO | FR1-FR1 | 47 | 5.27 | Note 2 |
| Source 10 | R1-2111528 | 10 | 0.40% | 99.0% | Typical HO | FR1-FR1 | 47 | 6.142 | Note 2 |
| Source 10 | R1-2111529 | 10 | 0.50% | 99.0% | Typical HO | FR1-FR1 | 47 | 7.363 | Note 2 |
| Source 10 | R1-2111530 | 10 | 0.60% | 99.0% | Typical HO | FR1-FR1 | 47 | 9.195 | Note 2 |
| Source 10 | R1-2111531 | 10 | 0.70% | 99.0% | Typical HO | FR1-FR1 | 47 | 12.25 | Note 2 |
| Source 10 | R1-2111532 | 10 | 0.80% | 99.0% | Typical HO | FR1-FR1 | 47 | 18.35 | Note 2 |
| Source 10 | R1-2111533 | 10 | 0.90% | 99.0% | Typical HO | FR1-FR1 | 47 | 36.67 | Note 2 |
| Source 10 | R1-2111524 | 10 | 0.01% | 99.0% | Typical HO | FR1-FR1 | 142 | 14.34 | Note 2 |
| Source 10 | R1-2111525 | 10 | 0.10% | 99.0% | Typical HO | FR1-FR1 | 142 | 15.76 | Note 2 |
| Source 10 | R1-2111526 | 10 | 0.20% | 99.0% | Typical HO | FR1-FR1 | 142 | 17.71 | Note 2 |
| Source 10 | R1-2111527 | 10 | 0.30% | 99.0% | Typical HO | FR1-FR1 | 142 | 20.22 | Note 2 |
| Source 10 | R1-2111528 | 10 | 0.40% | 99.0% | Typical HO | FR1-FR1 | 142 | 23.57 | Note 2 |
| Source 10 | R1-2111529 | 10 | 0.50% | 99.0% | Typical HO | FR1-FR1 | 142 | 28.83 | Note 2 |
| Source 10 | R1-2111530 | 10 | 0.60% | 99.0% | Typical HO | FR1-FR1 | 142 | 35.29 | Note 2 |
| Source 10 | R1-2111531 | 10 | 0.70% | 99.0% | Typical HO | FR1-FR1 | 142 | 47 | Note 2 |
| Source 10 | R1-2111532 | 10 | 0.80% | 99.0% | Typical HO | FR1-FR1 | 142 | 70.43 | Note 2 |
| Source 10 | R1-2111533 | 10 | 0.90% | 99.0% | Typical HO | FR1-FR1 | 142 | 140.7 | Note 2 |
| Source 13 | R1-2112069 | 10 | 0.0% | 99.0% |  |  | 40 | 3.0 |  |
| Source 13 | R1-2112069 | 10 | 0.0% | 99.0% |  |  | 42 | 3.2 |  |
| Source 13 | R1-2112069 | 10 | 0.0% | 99.0% |  |  | 44 | 3.4 |  |
| Source 13 | R1-2112069 | 10 | 0.0% | 99.0% |  |  | 46 | 3.6 |  |
| Source 13 | R1-2112069 | 10 | 0.0% | 99.0% |  |  | 48 | 3.8 |  |
| Source 13 | R1-2112069 | 10 | 0.0% | 99.0% |  |  | 50 | 4.0 |  |
| Source 13 | R1-2112069 | 10 | 0.0% | 99.0% |  |  | 52 | 4.2 |  |
| Source 13 | R1-2112069 | 10 | 0.0% | 99.0% |  |  | 54 | 4.4 |  |
| Source 13 | R1-2112069 | 10 | 0.0% | 99.0% |  |  | 56 | 4.6 |  |
| Source 13 | R1-2112069 | 10 | 0.0% | 99.0% |  |  | 58 | 4.8 |  |
| Source 13 | R1-2112069 | 10 | 0.0% | 99.0% |  |  | 60 | 5.0 |  |
| Source 13 | R1-2112069 | 10 | 0.0% | 99.0% |  |  | 62 | 5.2 |  |
| Source 13 | R1-2112069 | 10 | 0.0% | 99.0% |  |  | 64 | 5.4 |  |
| Source 13 | R1-2112069 | 10 | 0.0% | 99.0% |  |  | 66 | 5.6 |  |
| Source 13 | R1-2112069 | 10 | 0.0% | 99.0% |  |  | 68 | 5.8 |  |
| Source 13 | R1-2112069 | 10 | 0.0% | 99.0% |  |  | 70 | 6.0 |  |
| Source 13 | R1-2112069 | 10 | 0.0% | 99.0% |  |  | 72 | 6.2 |  |
| Source 13 | R1-2112069 | 10 | 0.0% | 99.0% |  |  | 74 | 6.4 |  |
| Source 13 | R1-2112069 | 10 | 0.0% | 99.0% |  |  | 76 | 6.6 |  |
| Source 13 | R1-2112069 | 10 | 0.0% | 99.0% |  |  | 78 | 6.8 |  |
| Source 13 | R1-2112069 | 10 | 0.0% | 99.0% |  |  | 80 | 7.0 |  |
| Source 13 | R1-2112069 | 10 | 0.5% | 99.0% |  |  | 40 | 6.0 |  |
| Source 13 | R1-2112069 | 10 | 0.5% | 99.0% |  |  | 42 | 6.4 |  |
| Source 13 | R1-2112069 | 10 | 0.5% | 99.0% |  |  | 44 | 6.8 |  |
| Source 13 | R1-2112069 | 10 | 0.5% | 99.0% |  |  | 46 | 7.2 |  |
| Source 13 | R1-2112069 | 10 | 0.5% | 99.0% |  |  | 48 | 7.6 |  |
| Source 13 | R1-2112069 | 10 | 0.5% | 99.0% |  |  | 50 | 8.0 |  |
| Source 13 | R1-2112069 | 10 | 0.5% | 99.0% |  |  | 52 | 8.4 |  |
| Source 13 | R1-2112069 | 10 | 0.5% | 99.0% |  |  | 54 | 8.8 |  |
| Source 13 | R1-2112069 | 10 | 0.5% | 99.0% |  |  | 56 | 9.2 |  |
| Source 13 | R1-2112069 | 10 | 0.5% | 99.0% |  |  | 58 | 9.6 |  |
| Source 13 | R1-2112069 | 10 | 0.5% | 99.0% |  |  | 60 | 10.0 |  |
| Source 13 | R1-2112069 | 10 | 0.5% | 99.0% |  |  | 62 | 10.3 |  |
| Source 13 | R1-2112069 | 10 | 0.5% | 99.0% |  |  | 64 | 10.7 |  |
| Source 13 | R1-2112069 | 10 | 0.5% | 99.0% |  |  | 66 | 11.1 |  |
| Source 13 | R1-2112069 | 10 | 0.5% | 99.0% |  |  | 68 | 11.5 |  |
| Source 13 | R1-2112069 | 10 | 0.5% | 99.0% |  |  | 70 | 11.9 |  |
| Source 13 | R1-2112069 | 10 | 0.5% | 99.0% |  |  | 72 | 12.3 |  |
| Source 13 | R1-2112069 | 10 | 0.5% | 99.0% |  |  | 74 | 12.7 |  |
| Source 13 | R1-2112069 | 10 | 0.5% | 99.0% |  |  | 76 | 13.1 |  |
| Source 13 | R1-2112069 | 10 | 0.5% | 99.0% |  |  | 78 | 13.5 |  |
| Source 13 | R1-2112069 | 10 | 0.5% | 99.0% |  |  | 80 | 13.9 |  |
| Source 13 | R1-2112069 | 10 | 0.00% | 99.0% |  |  | 40 | 3.0 |  |
| Source 13 | R1-2112069 | 10 | 0.05% | 99.0% |  |  | 40 | 3.2 |  |
| Source 13 | R1-2112069 | 10 | 0.10% | 99.0% |  |  | 40 | 3.3 |  |
| Source 13 | R1-2112069 | 10 | 0.15% | 99.0% |  |  | 40 | 3.5 |  |
| Source 13 | R1-2112069 | 10 | 0.20% | 99.0% |  |  | 40 | 3.7 |  |
| Source 13 | R1-2112069 | 10 | 0.25% | 99.0% |  |  | 40 | 4.0 |  |
| Source 13 | R1-2112069 | 10 | 0.30% | 99.0% |  |  | 40 | 4.3 |  |
| Source 13 | R1-2112069 | 10 | 0.35% | 99.0% |  |  | 40 | 4.6 |  |
| Source 13 | R1-2112069 | 10 | 0.40% | 99.0% |  |  | 40 | 5.0 |  |
| Source 13 | R1-2112069 | 10 | 0.45% | 99.0% |  |  | 40 | 5.4 |  |
| Source 13 | R1-2112069 | 10 | 0.50% | 99.0% |  |  | 40 | 6.0 |  |
| Source 13 | R1-2112069 | 10 | 0.55% | 99.0% |  |  | 40 | 6.6 |  |
| Source 13 | R1-2112069 | 10 | 0.60% | 99.0% |  |  | 40 | 7.5 |  |
| Source 13 | R1-2112069 | 10 | 0.65% | 99.0% |  |  | 40 | 8.5 |  |
| Source 13 | R1-2112069 | 10 | 0.70% | 99.0% |  |  | 40 | 9.9 |  |
| Source 13 | R1-2112069 | 10 | 0.75% | 99.0% |  |  | 40 | 11.9 |  |
| Source 13 | R1-2112069 | 10 | 0.80% | 99.0% |  |  | 40 | 14.9 |  |
| Source 13 | R1-2112069 | 10 | 0.85% | 99.0% |  |  | 40 | 19.8 |  |
| Source 13 | R1-2112069 | 10 | 0.90% | 99.0% |  |  | 40 | 29.7 |  |
| Source 13 | R1-2112069 | 10 | 0.95% | 99.0% |  |  | 40 | 59.4 |  |
| Source 20 | R1-2111351 | 10 | 0.10% | 99.0% | DAPS | FR1-FR1 | 0 | 0 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.10% | 99.0% | DAPS | FR1-FR1 | 2 | 0.22 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.10% | 99.0% | DAPS | FR1-FR1 | 4 | 0.44 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.10% | 99.0% | DAPS | FR1-FR1 | 6 | 0.67 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.10% | 99.0% | DAPS | FR1-FR1 | 8 | 0.89 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.10% | 99.0% | DAPS | FR1-FR1 | 10 | 1.11 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.10% | 99.0% | Typical HO | FR1-FR1 | 30 | 2.22 |  |
| Source 20 | R1-2111351 | 10 | 0.10% | 99.0% | Conditional HO | FR1-FR1 | 40 | 3.33 |  |
| Source 20 | R1-2111351 | 10 | 0.10% | 99.0% | Typical HO | FR1-FR1 | 50 | 4.44 |  |
| Source 20 | R1-2111351 | 10 | 0.10% | 99.0% | Conditional HO | FR1-FR1 | 60 | 5.55 |  |
| Source 20 | R1-2111351 | 10 | 0.10% | 99.0% | Typical HO | FR1-FR1 | 70 | 6.66 |  |
| Source 20 | R1-2111351 | 10 | 0.20% | 99.0% | DAPS | FR1-FR1 | 0 | 0 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.20% | 99.0% | DAPS | FR1-FR1 | 2 | 0.25 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.20% | 99.0% | DAPS | FR1-FR1 | 4 | 0.5 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.20% | 99.0% | DAPS | FR1-FR1 | 6 | 0.75 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.20% | 99.0% | DAPS | FR1-FR1 | 8 | 1 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.20% | 99.0% | DAPS | FR1-FR1 | 10 | 1.25 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.20% | 99.0% | Typical HO | FR1-FR1 | 30 | 2.5 |  |
| Source 20 | R1-2111351 | 10 | 0.20% | 99.0% | Conditional HO | FR1-FR1 | 40 | 3.74 |  |
| Source 20 | R1-2111351 | 10 | 0.20% | 99.0% | Typical HO | FR1-FR1 | 50 | 4.99 |  |
| Source 20 | R1-2111351 | 10 | 0.20% | 99.0% | Conditional HO | FR1-FR1 | 60 | 6.24 |  |
| Source 20 | R1-2111351 | 10 | 0.20% | 99.0% | Typical HO | FR1-FR1 | 70 | 7.49 |  |
| Source 20 | R1-2111351 | 10 | 0.30% | 99.0% | DAPS | FR1-FR1 | 0 | 0 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.30% | 99.0% | DAPS | FR1-FR1 | 2 | 0.28 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.30% | 99.0% | DAPS | FR1-FR1 | 4 | 0.57 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.30% | 99.0% | DAPS | FR1-FR1 | 6 | 0.85 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.30% | 99.0% | DAPS | FR1-FR1 | 8 | 1.14 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.30% | 99.0% | DAPS | FR1-FR1 | 10 | 1.42 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.30% | 99.0% | Typical HO | FR1-FR1 | 30 | 2.85 |  |
| Source 20 | R1-2111351 | 10 | 0.30% | 99.0% | Conditional HO | FR1-FR1 | 40 | 4.27 |  |
| Source 20 | R1-2111351 | 10 | 0.30% | 99.0% | Typical HO | FR1-FR1 | 50 | 5.7 |  |
| Source 20 | R1-2111351 | 10 | 0.30% | 99.0% | Conditional HO | FR1-FR1 | 60 | 7.12 |  |
| Source 20 | R1-2111351 | 10 | 0.30% | 99.0% | Typical HO | FR1-FR1 | 70 | 8.55 |  |
| Source 20 | R1-2111351 | 10 | 0.40% | 99.0% | DAPS | FR1-FR1 | 0 | 0 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.40% | 99.0% | DAPS | FR1-FR1 | 2 | 0.32 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.40% | 99.0% | DAPS | FR1-FR1 | 4 | 0.66 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.40% | 99.0% | DAPS | FR1-FR1 | 6 | 1 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.40% | 99.0% | DAPS | FR1-FR1 | 8 | 1.33 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.40% | 99.0% | DAPS | FR1-FR1 | 10 | 1.66 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.40% | 99.0% | Typical HO | FR1-FR1 | 30 | 3.32 |  |
| Source 20 | R1-2111351 | 10 | 0.40% | 99.0% | Conditional HO | FR1-FR1 | 40 | 4.98 |  |
| Source 20 | R1-2111351 | 10 | 0.40% | 99.0% | Typical HO | FR1-FR1 | 50 | 6.64 |  |
| Source 20 | R1-2111351 | 10 | 0.40% | 99.0% | Conditional HO | FR1-FR1 | 60 | 8.30 |  |
| Source 20 | R1-2111351 | 10 | 0.40% | 99.0% | Typical HO | FR1-FR1 | 70 | 9.96 |  |
| Source 20 | R1-2111351 | 10 | 0.50% | 99.0% | DAPS | FR1-FR1 | 0 | 0 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.50% | 99.0% | DAPS | FR1-FR1 | 2 | 0.4 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.50% | 99.0% | DAPS | FR1-FR1 | 4 | 0.8 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.50% | 99.0% | DAPS | FR1-FR1 | 6 | 1.2 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.50% | 99.0% | DAPS | FR1-FR1 | 8 | 1.59 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.50% | 99.0% | DAPS | FR1-FR1 | 10 | 1.99 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.50% | 99.0% | Typical HO | FR1-FR1 | 30 | 3.98 |  |
| Source 20 | R1-2111351 | 10 | 0.50% | 99.0% | Conditional HO | FR1-FR1 | 40 | 5.97 |  |
| Source 20 | R1-2111351 | 10 | 0.50% | 99.0% | Typical HO | FR1-FR1 | 50 | 7.96 |  |
| Source 20 | R1-2111351 | 10 | 0.50% | 99.0% | Conditional HO | FR1-FR1 | 60 | 9.95 |  |
| Source 20 | R1-2111351 | 10 | 0.50% | 99.0% | Typical HO | FR1-FR1 | 70 | 11.94 |  |
| Source 20 | R1-2111351 | 10 | 0.60% | 99.0% | DAPS | FR1-FR1 | 0 | 0 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.60% | 99.0% | DAPS | FR1-FR1 | 2 | 0.5 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.60% | 99.0% | DAPS | FR1-FR1 | 4 | 0.99 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.60% | 99.0% | DAPS | FR1-FR1 | 6 | 1.49 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.60% | 99.0% | DAPS | FR1-FR1 | 8 | 1.99 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.60% | 99.0% | DAPS | FR1-FR1 | 10 | 2.49 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.60% | 99.0% | Typical HO | FR1-FR1 | 30 | 4.97 |  |
| Source 20 | R1-2111351 | 10 | 0.60% | 99.0% | Conditional HO | FR1-FR1 | 40 | 7.46 |  |
| Source 20 | R1-2111351 | 10 | 0.60% | 99.0% | Typical HO | FR1-FR1 | 50 | 9.94 |  |
| Source 20 | R1-2111351 | 10 | 0.60% | 99.0% | Conditional HO | FR1-FR1 | 60 | 12.43 |  |
| Source 20 | R1-2111351 | 10 | 0.60% | 99.0% | Typical HO | FR1-FR1 | 70 | 14.91 |  |
| Source 20 | R1-2111351 | 10 | 0.70% | 99.0% | DAPS | FR1-FR1 | 0 | 0 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.70% | 99.0% | DAPS | FR1-FR1 | 2 | 0.66 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.70% | 99.0% | DAPS | FR1-FR1 | 4 | 1.32 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.70% | 99.0% | DAPS | FR1-FR1 | 6 | 1.99 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.70% | 99.0% | DAPS | FR1-FR1 | 8 | 2.65 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.70% | 99.0% | DAPS | FR1-FR1 | 10 | 3.31 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.70% | 99.0% | Typical HO | FR1-FR1 | 30 | 6.62 |  |
| Source 20 | R1-2111351 | 10 | 0.70% | 99.0% | Conditional HO | FR1-FR1 | 40 | 9.93 |  |
| Source 20 | R1-2111351 | 10 | 0.70% | 99.0% | Typical HO | FR1-FR1 | 50 | 13.24 |  |
| Source 20 | R1-2111351 | 10 | 0.70% | 99.0% | Conditional HO | FR1-FR1 | 60 | 16.55 |  |
| Source 20 | R1-2111351 | 10 | 0.70% | 99.0% | Typical HO | FR1-FR1 | 70 | 19.86 |  |
| Source 20 | R1-2111351 | 10 | 0.80% | 99.0% | DAPS | FR1-FR1 | 0 | 0 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.80% | 99.0% | DAPS | FR1-FR1 | 2 | 0.99 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.80% | 99.0% | DAPS | FR1-FR1 | 4 | 1.98 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.80% | 99.0% | DAPS | FR1-FR1 | 6 | 2.98 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.80% | 99.0% | DAPS | FR1-FR1 | 8 | 3.97 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.80% | 99.0% | DAPS | FR1-FR1 | 10 | 4.96 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.80% | 99.0% | Typical HO | FR1-FR1 | 30 | 9.92 |  |
| Source 20 | R1-2111351 | 10 | 0.80% | 99.0% | Conditional HO | FR1-FR1 | 40 | 14.88 |  |
| Source 20 | R1-2111351 | 10 | 0.80% | 99.0% | Typical HO | FR1-FR1 | 50 | 19.84 |  |
| Source 20 | R1-2111351 | 10 | 0.80% | 99.0% | Conditional HO | FR1-FR1 | 60 | 24.80 |  |
| Source 20 | R1-2111351 | 10 | 0.80% | 99.0% | Typical HO | FR1-FR1 | 70 | 29.76 |  |
| Source 20 | R1-2111351 | 10 | 0.90% | 99.0% | DAPS | FR1-FR1 | 0 | 0 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.90% | 99.0% | DAPS | FR1-FR1 | 2 | 1.98 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.90% | 99.0% | DAPS | FR1-FR1 | 4 | 3.96 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.90% | 99.0% | DAPS | FR1-FR1 | 6 | 5.96 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.90% | 99.0% | DAPS | FR1-FR1 | 8 | 7.93 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.90% | 99.0% | DAPS | FR1-FR1 | 10 | 9.91 | Note 5 |
| Source 20 | R1-2111351 | 10 | 0.90% | 99.0% | Typical HO | FR1-FR1 | 30 | 19.82 |  |
| Source 20 | R1-2111351 | 10 | 0.90% | 99.0% | Conditional HO | FR1-FR1 | 40 | 29.73 |  |
| Source 20 | R1-2111351 | 10 | 0.90% | 99.0% | Typical HO | FR1-FR1 | 50 | 39.64 |  |
| Source 20 | R1-2111351 | 10 | 0.90% | 99.0% | Conditional HO | FR1-FR1 | 60 | 49.55 |  |
| Source 20 | R1-2111351 | 10 | 0.90% | 99.0% | Typical HO | FR1-FR1 | 70 | 59.46 |  |
| Note 1: the target cell is known Note 2: target cell is an unknown intra-frequency cell Note 3: target cell is an unknown inter-frequency cell Note 4: T = 0, Y < PDB Note 5: T = Y * (100%-PE,op) / (100%-X- PE,op), Y < PDB |  |  |  |  |  |  |  |  |  |

Table B.2-2. X=99%, PDB=15ms

| Company | Tdoc | PDB (ms) | PE,op | X | Handover type | Handover case | Y (ms) | T (s) | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | R1-2111046 | 15 | 0.00% | 99.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 3.70 |  |
| Source 18 | R1-2111046 | 15 | 0.00% | 99.0% | Typical HO | FR1-to-FR1 | 62 | 4.70 |  |
| Source 18 | R1-2111046 | 15 | 0.00% | 99.0% | Typical HO | FR1-to-FR1 | 82 | 6.70 |  |
| Source 18 | R1-2111046 | 15 | 0.10% | 99.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 4.11 |  |
| Source 18 | R1-2111046 | 15 | 0.10% | 99.0% | Typical HO | FR1-to-FR1 | 62 | 5.22 |  |
| Source 18 | R1-2111046 | 15 | 0.10% | 99.0% | Typical HO | FR1-to-FR1 | 82 | 7.44 |  |
| Source 18 | R1-2111046 | 15 | 0.50% | 99.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 7.36 |  |
| Source 18 | R1-2111046 | 15 | 0.50% | 99.0% | Typical HO | FR1-to-FR1 | 62 | 9.35 |  |
| Source 18 | R1-2111046 | 15 | 0.50% | 99.0% | Typical HO | FR1-to-FR1 | 82 | 13.33 |  |
| Source 18 | R1-2111046 | 15 | 0.90% | 99.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 36.67 |  |
| Source 18 | R1-2111046 | 15 | 0.90% | 99.0% | Typical HO | FR1-to-FR1 | 62 | 46.58 |  |
| Source 18 | R1-2111046 | 15 | 0.90% | 99.0% | Typical HO | FR1-to-FR1 | 82 | 66.40 |  |
| Source 3 | R1-2111234 | 15 | 0.01% | 99.0% | Typical HO | FR1-to-FR1 | 66.5 | 4.192 |  |
| Source 3 | R1-2111234 | 15 | 0.01% | 99.0% | Typical HO | FR2-to-FR2 | 59.4 | 3.474 |  |
| Source 3 | R1-2111234 | 15 | 0.07% | 99.0% | Typical HO | FR1-to-FR1 | 66.5 | 4.459 |  |
| Source 3 | R1-2111234 | 15 | 0.07% | 99.0% | Typical HO | FR2-to-FR2 | 59.4 | 3.696 |  |
| Source 3 | R1-2111234 | 15 | 0.25% | 99.0% | Typical HO | FR1-to-FR1 | 66.5 | 5.52 |  |
| Source 3 | R1-2111234 | 15 | 0.25% | 99.0% | Typical HO | FR2-to-FR2 | 59.4 | 4.575 |  |
| Source 3 | R1-2111234 | 15 | 0.01% | 99.0% | Conditional HO | FR1-to-FR1 | 56.5 | 4.192 |  |
| Source 3 | R1-2111234 | 15 | 0.01% | 99.0% | Conditional HO | FR2-to-FR2 | 49.4 | 3.474 |  |
| Source 3 | R1-2111234 | 15 | 0.07% | 99.0% | Conditional HO | FR1-to-FR1 | 56.5 | 4.277 |  |
| Source 3 | R1-2111234 | 15 | 0.07% | 99.0% | Conditional HO | FR2-to-FR2 | 49.4 | 3.545 |  |
| Source 3 | R1-2111234 | 15 | 0.25% | 99.0% | Conditional HO | FR1-to-FR1 | 56.5 | 5.996 |  |
| Source 3 | R1-2111234 | 15 | 0.25% | 99.0% | Conditional HO | FR2-to-FR2 | 49.4 | 4.97 |  |
| Source 6 | R1-2111632 | 15 | 0.10% | 99.0% |  |  | 48 | 3.663 |  |
| Source 15 | R1-2111828 | 15 | 0% | 99.0% | DAPS |  | 2 | 0 | Note 1 |
| Source 15 | R1-2111828 | 15 | 0% | 99.0% | Typical HO, Conditional HO |  | 42 | 2.7 |  |
| Source 15 | R1-2111828 | 15 | 0% | 99.0% | Typical HO, Conditional HO |  | 62 | 4.7 |  |
| Source 15 | R1-2111828 | 15 | 0% | 99.0% |  |  | 10 | 0 | Note 1 |
| Source 15 | R1-2111828 | 15 | 0% | 99.0% |  |  | 20 | 0.5 |  |
| Source 15 | R1-2111828 | 15 | 0% | 99.0% |  |  | 40 | 2.5 |  |
| Source 15 | R1-2111828 | 15 | 0% | 99.0% |  |  | 60 | 4.5 |  |
| Source 15 | R1-2111828 | 15 | 0% | 99.0% |  |  | 80 | 6.5 |  |
| Source 15 | R1-2111828 | 15 | 0% | 99.0% |  |  | 100 | 8.5 |  |
| Source 15 | R1-2111828 | 15 | 0.20% | 99.0% | DAPS |  | 2 | 0.00 | Note 1 |
| Source 15 | R1-2111828 | 15 | 0.20% | 99.0% | Typical HO, Conditional HO |  | 42 | 3.37 |  |
| Source 15 | R1-2111828 | 15 | 0.20% | 99.0% | Typical HO, Conditional HO |  | 62 | 5.86 |  |
| Source 15 | R1-2111828 | 15 | 0.20% | 99.0% |  |  | 10 | 0.00 | Note 1 |
| Source 15 | R1-2111828 | 15 | 0.20% | 99.0% |  |  | 20 | 0.62 |  |
| Source 15 | R1-2111828 | 15 | 0.20% | 99.0% |  |  | 40 | 3.12 |  |
| Source 15 | R1-2111828 | 15 | 0.20% | 99.0% |  |  | 60 | 5.61 |  |
| Source 15 | R1-2111828 | 15 | 0.20% | 99.0% |  |  | 80 | 8.11 |  |
| Source 15 | R1-2111828 | 15 | 0.20% | 99.0% |  |  | 100 | 10.60 |  |
| Source 15 | R1-2111828 | 15 | 0.40% | 99.0% | DAPS |  | 2 | 0.00 | Note 1 |
| Source 15 | R1-2111828 | 15 | 0.40% | 99.0% | Typical HO, Conditional HO |  | 42 | 4.48 |  |
| Source 15 | R1-2111828 | 15 | 0.40% | 99.0% | Typical HO, Conditional HO |  | 62 | 7.80 |  |
| Source 15 | R1-2111828 | 15 | 0.40% | 99.0% |  |  | 10 | 0.00 | Note 1 |
| Source 15 | R1-2111828 | 15 | 0.40% | 99.0% |  |  | 20 | 0.83 |  |
| Source 15 | R1-2111828 | 15 | 0.40% | 99.0% |  |  | 40 | 4.15 |  |
| Source 15 | R1-2111828 | 15 | 0.40% | 99.0% |  |  | 60 | 7.47 |  |
| Source 15 | R1-2111828 | 15 | 0.40% | 99.0% |  |  | 80 | 10.79 |  |
| Source 15 | R1-2111828 | 15 | 0.40% | 99.0% |  |  | 100 | 14.11 |  |
| Source 15 | R1-2111828 | 15 | 0.60% | 99.0% | DAPS |  | 2 | 0.00 | Note 1 |
| Source 15 | R1-2111828 | 15 | 0.60% | 99.0% | Typical HO, Conditional HO |  | 42 | 6.71 |  |
| Source 15 | R1-2111828 | 15 | 0.60% | 99.0% | Typical HO, Conditional HO |  | 62 | 11.68 |  |
| Source 15 | R1-2111828 | 15 | 0.60% | 99.0% |  |  | 10 | 0.00 | Note 1 |
| Source 15 | R1-2111828 | 15 | 0.60% | 99.0% |  |  | 20 | 1.24 |  |
| Source 15 | R1-2111828 | 15 | 0.60% | 99.0% |  |  | 40 | 6.21 |  |
| Source 15 | R1-2111828 | 15 | 0.60% | 99.0% |  |  | 60 | 11.18 |  |
| Source 15 | R1-2111828 | 15 | 0.60% | 99.0% |  |  | 80 | 16.15 |  |
| Source 15 | R1-2111828 | 15 | 0.60% | 99.0% |  |  | 100 | 21.12 |  |
| Source 15 | R1-2111828 | 15 | 0.80% | 99.0% | DAPS |  | 2 | 0.00 | Note 1 |
| Source 15 | R1-2111828 | 15 | 0.80% | 99.0% | Typical HO, Conditional HO |  | 42 | 13.39 |  |
| Source 15 | R1-2111828 | 15 | 0.80% | 99.0% | Typical HO, Conditional HO |  | 62 | 23.31 |  |
| Source 15 | R1-2111828 | 15 | 0.80% | 99.0% |  |  | 10 | 0.00 | Note 1 |
| Source 15 | R1-2111828 | 15 | 0.80% | 99.0% |  |  | 20 | 2.48 |  |
| Source 15 | R1-2111828 | 15 | 0.80% | 99.0% |  |  | 40 | 12.40 |  |
| Source 15 | R1-2111828 | 15 | 0.80% | 99.0% |  |  | 60 | 22.32 |  |
| Source 15 | R1-2111828 | 15 | 0.80% | 99.0% |  |  | 80 | 32.24 |  |
| Source 15 | R1-2111828 | 15 | 0.80% | 99.0% |  |  | 100 | 42.16 |  |
| Source 15 | R1-2111828 | 15 | 0.90% | 99.0% | DAPS |  | 2 | 0.00 | Note 1 |
| Source 15 | R1-2111828 | 15 | 0.90% | 99.0% | Typical HO, Conditional HO |  | 42 | 26.76 |  |
| Source 15 | R1-2111828 | 15 | 0.90% | 99.0% | Typical HO, Conditional HO |  | 62 | 46.58 |  |
| Source 15 | R1-2111828 | 15 | 0.90% | 99.0% |  |  | 10 | 0.00 | Note 1 |
| Source 15 | R1-2111828 | 15 | 0.90% | 99.0% |  |  | 20 | 4.95 |  |
| Source 15 | R1-2111828 | 15 | 0.90% | 99.0% |  |  | 40 | 24.77 |  |
| Source 15 | R1-2111828 | 15 | 0.90% | 99.0% |  |  | 60 | 44.59 |  |
| Source 15 | R1-2111828 | 15 | 0.90% | 99.0% |  |  | 80 | 64.41 |  |
| Source 15 | R1-2111828 | 15 | 0.90% | 99.0% |  |  | 100 | 84.23 |  |
| Source 7 | R1-2112160 | 15 | 0.00% | 99.0% | DAPS |  | 2 | 0.0 | Note 1 |
| Source 7 | R1-2112160 | 15 | 0.00% | 99.0% |  |  | 20 | 0.5 |  |
| Source 7 | R1-2112160 | 15 | 0.00% | 99.0% | Typical HO, Conditional HO |  | 40 | 2.5 |  |
| Source 7 | R1-2112160 | 15 | 0.00% | 99.0% |  |  | 60 | 4.5 |  |
| Source 7 | R1-2112160 | 15 | 0.00% | 99.0% |  |  | 80 | 6.5 |  |
| Source 7 | R1-2112160 | 15 | 0.00% | 99.0% |  |  | 100 | 8.5 |  |
| Source 7 | R1-2112160 | 15 | 0.20% | 99.0% | DAPS |  | 2 | 0.0 | Note 1 |
| Source 7 | R1-2112160 | 15 | 0.20% | 99.0% |  |  | 20 | 0.6 |  |
| Source 7 | R1-2112160 | 15 | 0.20% | 99.0% | Typical HO, Conditional HO |  | 40 | 3.1 |  |
| Source 7 | R1-2112160 | 15 | 0.20% | 99.0% |  |  | 60 | 5.6 |  |
| Source 7 | R1-2112160 | 15 | 0.20% | 99.0% |  |  | 80 | 8.1 |  |
| Source 7 | R1-2112160 | 15 | 0.20% | 99.0% |  |  | 100 | 10.6 |  |
| Source 7 | R1-2112160 | 15 | 0.40% | 99.0% | DAPS |  | 2 | 0.0 | Note 1 |
| Source 7 | R1-2112160 | 15 | 0.40% | 99.0% |  |  | 20 | 0.8 |  |
| Source 7 | R1-2112160 | 15 | 0.40% | 99.0% | Typical HO, Conditional HO |  | 40 | 4.1 |  |
| Source 7 | R1-2112160 | 15 | 0.40% | 99.0% |  |  | 60 | 7.5 |  |
| Source 7 | R1-2112160 | 15 | 0.40% | 99.0% |  |  | 80 | 10.8 |  |
| Source 7 | R1-2112160 | 15 | 0.40% | 99.0% |  |  | 100 | 14.1 |  |
| Source 7 | R1-2112160 | 15 | 0.60% | 99.0% | DAPS |  | 2 | 0.0 | Note 1 |
| Source 7 | R1-2112160 | 15 | 0.60% | 99.0% |  |  | 20 | 1.2 |  |
| Source 7 | R1-2112160 | 15 | 0.60% | 99.0% | Typical HO, Conditional HO |  | 40 | 6.2 |  |
| Source 7 | R1-2112160 | 15 | 0.60% | 99.0% |  |  | 60 | 11.2 |  |
| Source 7 | R1-2112160 | 15 | 0.60% | 99.0% |  |  | 80 | 16.2 |  |
| Source 7 | R1-2112160 | 15 | 0.60% | 99.0% |  |  | 100 | 21.1 |  |
| Source 7 | R1-2112160 | 15 | 0.80% | 99.0% | DAPS |  | 2 | 0.0 | Note 1 |
| Source 7 | R1-2112160 | 15 | 0.80% | 99.0% |  |  | 20 | 2.5 |  |
| Source 7 | R1-2112160 | 15 | 0.80% | 99.0% | Typical HO, Conditional HO |  | 40 | 12.4 |  |
| Source 7 | R1-2112160 | 15 | 0.80% | 99.0% |  |  | 60 | 22.3 |  |
| Source 7 | R1-2112160 | 15 | 0.80% | 99.0% |  |  | 80 | 32.2 |  |
| Source 7 | R1-2112160 | 15 | 0.80% | 99.0% |  |  | 100 | 42.2 |  |
| Source 7 | R1-2112160 | 15 | 0.90% | 99.0% | DAPS |  | 2 | 0.0 | Note 1 |
| Source 7 | R1-2112160 | 15 | 0.90% | 99.0% |  |  | 20 | 5.0 |  |
| Source 7 | R1-2112160 | 15 | 0.90% | 99.0% | Typical HO, Conditional HO |  | 40 | 24.8 |  |
| Source 7 | R1-2112160 | 15 | 0.90% | 99.0% |  |  | 60 | 44.6 |  |
| Source 7 | R1-2112160 | 15 | 0.90% | 99.0% |  |  | 80 | 64.4 |  |
| Source 7 | R1-2112160 | 15 | 0.90% | 99.0% |  |  | 100 | 84.2 |  |
| Source 14 | R1-2112296 | 15 | 0.0% | 99.0% |  |  | 60 | 4.5 |  |
| Source 14 | R1-2112296 | 15 | 0.5% | 99.0% |  |  | 60 | 8.955 |  |
| Source 14 | R1-2112296 | 15 | 0.9% | 99.0% |  |  | 60 | 44.595 |  |
| Source 13 | R1-2112069 | 15 | 0.0% | 99.0% |  |  | 40 | 2.5 |  |
| Source 13 | R1-2112069 | 15 | 0.0% | 99.0% |  |  | 42 | 2.7 |  |
| Source 13 | R1-2112069 | 15 | 0.0% | 99.0% |  |  | 44 | 2.9 |  |
| Source 13 | R1-2112069 | 15 | 0.0% | 99.0% |  |  | 46 | 3.1 |  |
| Source 13 | R1-2112069 | 15 | 0.0% | 99.0% |  |  | 48 | 3.3 |  |
| Source 13 | R1-2112069 | 15 | 0.0% | 99.0% |  |  | 50 | 3.5 |  |
| Source 13 | R1-2112069 | 15 | 0.0% | 99.0% |  |  | 52 | 3.7 |  |
| Source 13 | R1-2112069 | 15 | 0.0% | 99.0% |  |  | 54 | 3.9 |  |
| Source 13 | R1-2112069 | 15 | 0.0% | 99.0% |  |  | 56 | 4.1 |  |
| Source 13 | R1-2112069 | 15 | 0.0% | 99.0% |  |  | 58 | 4.3 |  |
| Source 13 | R1-2112069 | 15 | 0.0% | 99.0% |  |  | 60 | 4.5 |  |
| Source 13 | R1-2112069 | 15 | 0.0% | 99.0% |  |  | 62 | 4.7 |  |
| Source 13 | R1-2112069 | 15 | 0.0% | 99.0% |  |  | 64 | 4.9 |  |
| Source 13 | R1-2112069 | 15 | 0.0% | 99.0% |  |  | 66 | 5.1 |  |
| Source 13 | R1-2112069 | 15 | 0.0% | 99.0% |  |  | 68 | 5.3 |  |
| Source 13 | R1-2112069 | 15 | 0.0% | 99.0% |  |  | 70 | 5.5 |  |
| Source 13 | R1-2112069 | 15 | 0.0% | 99.0% |  |  | 72 | 5.7 |  |
| Source 13 | R1-2112069 | 15 | 0.0% | 99.0% |  |  | 74 | 5.9 |  |
| Source 13 | R1-2112069 | 15 | 0.0% | 99.0% |  |  | 76 | 6.1 |  |
| Source 13 | R1-2112069 | 15 | 0.0% | 99.0% |  |  | 78 | 6.3 |  |
| Source 13 | R1-2112069 | 15 | 0.0% | 99.0% |  |  | 80 | 6.5 |  |
| Source 13 | R1-2112069 | 15 | 0.5% | 99.0% |  |  | 40 | 5.0 |  |
| Source 13 | R1-2112069 | 15 | 0.5% | 99.0% |  |  | 42 | 5.4 |  |
| Source 13 | R1-2112069 | 15 | 0.5% | 99.0% |  |  | 44 | 5.8 |  |
| Source 13 | R1-2112069 | 15 | 0.5% | 99.0% |  |  | 46 | 6.2 |  |
| Source 13 | R1-2112069 | 15 | 0.5% | 99.0% |  |  | 48 | 6.6 |  |
| Source 13 | R1-2112069 | 15 | 0.5% | 99.0% |  |  | 50 | 7.0 |  |
| Source 13 | R1-2112069 | 15 | 0.5% | 99.0% |  |  | 52 | 7.4 |  |
| Source 13 | R1-2112069 | 15 | 0.5% | 99.0% |  |  | 54 | 7.8 |  |
| Source 13 | R1-2112069 | 15 | 0.5% | 99.0% |  |  | 56 | 8.2 |  |
| Source 13 | R1-2112069 | 15 | 0.5% | 99.0% |  |  | 58 | 8.6 |  |
| Source 13 | R1-2112069 | 15 | 0.5% | 99.0% |  |  | 60 | 9.0 |  |
| Source 13 | R1-2112069 | 15 | 0.5% | 99.0% |  |  | 62 | 9.4 |  |
| Source 13 | R1-2112069 | 15 | 0.5% | 99.0% |  |  | 64 | 9.8 |  |
| Source 13 | R1-2112069 | 15 | 0.5% | 99.0% |  |  | 66 | 10.1 |  |
| Source 13 | R1-2112069 | 15 | 0.5% | 99.0% |  |  | 68 | 10.5 |  |
| Source 13 | R1-2112069 | 15 | 0.5% | 99.0% |  |  | 70 | 10.9 |  |
| Source 13 | R1-2112069 | 15 | 0.5% | 99.0% |  |  | 72 | 11.3 |  |
| Source 13 | R1-2112069 | 15 | 0.5% | 99.0% |  |  | 74 | 11.7 |  |
| Source 13 | R1-2112069 | 15 | 0.5% | 99.0% |  |  | 76 | 12.1 |  |
| Source 13 | R1-2112069 | 15 | 0.5% | 99.0% |  |  | 78 | 12.5 |  |
| Source 13 | R1-2112069 | 15 | 0.5% | 99.0% |  |  | 80 | 12.9 |  |
| Source 13 | R1-2112069 | 15 | 0.00% | 99.0% |  |  | 40 | 2.5 |  |
| Source 13 | R1-2112069 | 15 | 0.05% | 99.0% |  |  | 40 | 2.6 |  |
| Source 13 | R1-2112069 | 15 | 0.10% | 99.0% |  |  | 40 | 2.8 |  |
| Source 13 | R1-2112069 | 15 | 0.15% | 99.0% |  |  | 40 | 2.9 |  |
| Source 13 | R1-2112069 | 15 | 0.20% | 99.0% |  |  | 40 | 3.1 |  |
| Source 13 | R1-2112069 | 15 | 0.25% | 99.0% |  |  | 40 | 3.3 |  |
| Source 13 | R1-2112069 | 15 | 0.30% | 99.0% |  |  | 40 | 3.6 |  |
| Source 13 | R1-2112069 | 15 | 0.35% | 99.0% |  |  | 40 | 3.8 |  |
| Source 13 | R1-2112069 | 15 | 0.40% | 99.0% |  |  | 40 | 4.2 |  |
| Source 13 | R1-2112069 | 15 | 0.45% | 99.0% |  |  | 40 | 4.5 |  |
| Source 13 | R1-2112069 | 15 | 0.50% | 99.0% |  |  | 40 | 5.0 |  |
| Source 13 | R1-2112069 | 15 | 0.55% | 99.0% |  |  | 40 | 5.5 |  |
| Source 13 | R1-2112069 | 15 | 0.60% | 99.0% |  |  | 40 | 6.2 |  |
| Source 13 | R1-2112069 | 15 | 0.65% | 99.0% |  |  | 40 | 7.1 |  |
| Source 13 | R1-2112069 | 15 | 0.70% | 99.0% |  |  | 40 | 8.3 |  |
| Source 13 | R1-2112069 | 15 | 0.75% | 99.0% |  |  | 40 | 9.9 |  |
| Source 13 | R1-2112069 | 15 | 0.80% | 99.0% |  |  | 40 | 12.4 |  |
| Source 13 | R1-2112069 | 15 | 0.85% | 99.0% |  |  | 40 | 16.5 |  |
| Source 13 | R1-2112069 | 15 | 0.90% | 99.0% |  |  | 40 | 24.8 |  |
| Source 13 | R1-2112069 | 15 | 0.95% | 99.0% |  |  | 40 | 49.5 |  |
| Note 1: T = 0, Y < PDB |  |  |  |  |  |  |  |  |  |

Table B.2-3. X=99%, PDB=30ms

| Company | Tdoc | PDB (ms) | PE,op | X | Handover type | Handover case | Y (ms) | T (s) | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | R1-2111046 | 30 | 0.00% | 99.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 2.20 |  |
| Source 18 | R1-2111046 | 30 | 0.00% | 99.0% | Typical HO | FR1-to-FR1 | 62 | 3.20 |  |
| Source 18 | R1-2111046 | 30 | 0.00% | 99.0% | Typical HO | FR1-to-FR1 | 82 | 5.20 |  |
| Source 18 | R1-2111046 | 30 | 0.10% | 99.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 2.44 |  |
| Source 18 | R1-2111046 | 30 | 0.10% | 99.0% | Typical HO | FR1-to-FR1 | 62 | 3.55 |  |
| Source 18 | R1-2111046 | 30 | 0.10% | 99.0% | Typical HO | FR1-to-FR1 | 82 | 5.77 |  |
| Source 18 | R1-2111046 | 30 | 0.50% | 99.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 4.38 |  |
| Source 18 | R1-2111046 | 30 | 0.50% | 99.0% | Typical HO | FR1-to-FR1 | 62 | 6.37 |  |
| Source 18 | R1-2111046 | 30 | 0.50% | 99.0% | Typical HO | FR1-to-FR1 | 82 | 10.35 |  |
| Source 18 | R1-2111046 | 30 | 0.90% | 99.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 21.80 |  |
| Source 18 | R1-2111046 | 30 | 0.90% | 99.0% | Typical HO | FR1-to-FR1 | 62 | 31.71 |  |
| Source 18 | R1-2111046 | 30 | 0.90% | 99.0% | Typical HO | FR1-to-FR1 | 82 | 51.53 |  |
| Source 15 | R1-2111828 | 30 | 0% | 99.0% | DAPS |  | 2 | 0 | Note 1 |
| Source 15 | R1-2111828 | 30 | 0% | 99.0% | Typical HO, Conditional HO |  | 42 | 1.2 |  |
| Source 15 | R1-2111828 | 30 | 0% | 99.0% | Typical HO, Conditional HO |  | 62 | 3.2 |  |
| Source 15 | R1-2111828 | 30 | 0% | 99.0% |  |  | 10 | 0 | Note 1 |
| Source 15 | R1-2111828 | 30 | 0% | 99.0% |  |  | 20 | 0 | Note 1 |
| Source 15 | R1-2111828 | 30 | 0% | 99.0% |  |  | 40 | 1 |  |
| Source 15 | R1-2111828 | 30 | 0% | 99.0% |  |  | 60 | 3 |  |
| Source 15 | R1-2111828 | 30 | 0% | 99.0% |  |  | 80 | 5 |  |
| Source 15 | R1-2111828 | 30 | 0% | 99.0% |  |  | 100 | 7 |  |
| Source 15 | R1-2111828 | 30 | 0.20% | 99.0% | DAPS |  | 2 | 0.00 | Note 1 |
| Source 15 | R1-2111828 | 30 | 0.20% | 99.0% | Typical HO, Conditional HO |  | 42 | 1.50 |  |
| Source 15 | R1-2111828 | 30 | 0.20% | 99.0% | Typical HO, Conditional HO |  | 62 | 3.99 |  |
| Source 15 | R1-2111828 | 30 | 0.20% | 99.0% |  |  | 10 | 0.00 | Note 1 |
| Source 15 | R1-2111828 | 30 | 0.20% | 99.0% |  |  | 20 | 0.00 | Note 1 |
| Source 15 | R1-2111828 | 30 | 0.20% | 99.0% |  |  | 40 | 1.25 |  |
| Source 15 | R1-2111828 | 30 | 0.20% | 99.0% |  |  | 60 | 3.74 |  |
| Source 15 | R1-2111828 | 30 | 0.20% | 99.0% |  |  | 80 | 6.24 |  |
| Source 15 | R1-2111828 | 30 | 0.20% | 99.0% |  |  | 100 | 8.73 |  |
| Source 15 | R1-2111828 | 30 | 0.40% | 99.0% | DAPS |  | 2 | 0.00 | Note 1 |
| Source 15 | R1-2111828 | 30 | 0.40% | 99.0% | Typical HO, Conditional HO |  | 42 | 1.99 |  |
| Source 15 | R1-2111828 | 30 | 0.40% | 99.0% | Typical HO, Conditional HO |  | 62 | 5.31 |  |
| Source 15 | R1-2111828 | 30 | 0.40% | 99.0% |  |  | 10 | 0.00 | Note 1 |
| Source 15 | R1-2111828 | 30 | 0.40% | 99.0% |  |  | 20 | 0.00 | Note 1 |
| Source 15 | R1-2111828 | 30 | 0.40% | 99.0% |  |  | 40 | 1.66 |  |
| Source 15 | R1-2111828 | 30 | 0.40% | 99.0% |  |  | 60 | 4.98 |  |
| Source 15 | R1-2111828 | 30 | 0.40% | 99.0% |  |  | 80 | 8.30 |  |
| Source 15 | R1-2111828 | 30 | 0.40% | 99.0% |  |  | 100 | 11.62 |  |
| Source 15 | R1-2111828 | 30 | 0.60% | 99.0% | DAPS |  | 2 | 0.00 | Note 1 |
| Source 15 | R1-2111828 | 30 | 0.60% | 99.0% | Typical HO, Conditional HO |  | 42 | 2.98 |  |
| Source 15 | R1-2111828 | 30 | 0.60% | 99.0% | Typical HO, Conditional HO |  | 62 | 7.95 |  |
| Source 15 | R1-2111828 | 30 | 0.60% | 99.0% |  |  | 10 | 0.00 | Note 1 |
| Source 15 | R1-2111828 | 30 | 0.60% | 99.0% |  |  | 20 | 0.00 | Note 1 |
| Source 15 | R1-2111828 | 30 | 0.60% | 99.0% |  |  | 40 | 2.48 |  |
| Source 15 | R1-2111828 | 30 | 0.60% | 99.0% |  |  | 60 | 7.45 |  |
| Source 15 | R1-2111828 | 30 | 0.60% | 99.0% |  |  | 80 | 12.43 |  |
| Source 15 | R1-2111828 | 30 | 0.60% | 99.0% |  |  | 100 | 17.40 |  |
| Source 15 | R1-2111828 | 30 | 0.80% | 99.0% | DAPS |  | 2 | 0.00 | Note 1 |
| Source 15 | R1-2111828 | 30 | 0.80% | 99.0% | Typical HO, Conditional HO |  | 42 | 5.95 |  |
| Source 15 | R1-2111828 | 30 | 0.80% | 99.0% | Typical HO, Conditional HO |  | 62 | 15.87 |  |
| Source 15 | R1-2111828 | 30 | 0.80% | 99.0% |  |  | 10 | 0.00 | Note 1 |
| Source 15 | R1-2111828 | 30 | 0.80% | 99.0% |  |  | 20 | 0.00 | Note 1 |
| Source 15 | R1-2111828 | 30 | 0.80% | 99.0% |  |  | 40 | 4.96 |  |
| Source 15 | R1-2111828 | 30 | 0.80% | 99.0% |  |  | 60 | 14.88 |  |
| Source 15 | R1-2111828 | 30 | 0.80% | 99.0% |  |  | 80 | 24.80 |  |
| Source 15 | R1-2111828 | 30 | 0.80% | 99.0% |  |  | 100 | 34.72 |  |
| Source 15 | R1-2111828 | 30 | 0.90% | 99.0% | DAPS |  | 2 | 0.00 | Note 1 |
| Source 15 | R1-2111828 | 30 | 0.90% | 99.0% | Typical HO, Conditional HO |  | 42 | 11.89 |  |
| Source 15 | R1-2111828 | 30 | 0.90% | 99.0% | Typical HO, Conditional HO |  | 62 | 31.71 |  |
| Source 15 | R1-2111828 | 30 | 0.90% | 99.0% |  |  | 10 | 0.00 | Note 1 |
| Source 15 | R1-2111828 | 30 | 0.90% | 99.0% |  |  | 20 | 0.00 | Note 1 |
| Source 15 | R1-2111828 | 30 | 0.90% | 99.0% |  |  | 40 | 9.91 |  |
| Source 15 | R1-2111828 | 30 | 0.90% | 99.0% |  |  | 60 | 29.73 |  |
| Source 15 | R1-2111828 | 30 | 0.90% | 99.0% |  |  | 80 | 49.55 |  |
| Source 15 | R1-2111828 | 30 | 0.90% | 99.0% |  |  | 100 | 69.37 |  |
| Source 7 | R1-2112160 | 30 | 0.00% | 99.0% | DAPS |  | 2 | 0.0 | Note 1 |
| Source 7 | R1-2112160 | 30 | 0.00% | 99.0% |  |  | 20 | 0.0 | Note 1 |
| Source 7 | R1-2112160 | 30 | 0.00% | 99.0% | Typical HO, Conditional HO |  | 40 | 1.0 |  |
| Source 7 | R1-2112160 | 30 | 0.00% | 99.0% |  |  | 60 | 3.0 |  |
| Source 7 | R1-2112160 | 30 | 0.00% | 99.0% |  |  | 80 | 5.0 |  |
| Source 7 | R1-2112160 | 30 | 0.00% | 99.0% |  |  | 100 | 7.0 |  |
| Source 7 | R1-2112160 | 30 | 0.20% | 99.0% | DAPS |  | 2 | 0.0 | Note 1 |
| Source 7 | R1-2112160 | 30 | 0.20% | 99.0% |  |  | 20 | 0.0 | Note 1 |
| Source 7 | R1-2112160 | 30 | 0.20% | 99.0% | Typical HO, Conditional HO |  | 40 | 1.2 |  |
| Source 7 | R1-2112160 | 30 | 0.20% | 99.0% |  |  | 60 | 3.7 |  |
| Source 7 | R1-2112160 | 30 | 0.20% | 99.0% |  |  | 80 | 6.2 |  |
| Source 7 | R1-2112160 | 30 | 0.20% | 99.0% |  |  | 100 | 8.7 |  |
| Source 7 | R1-2112160 | 30 | 0.40% | 99.0% | DAPS |  | 2 | 0.0 | Note 1 |
| Source 7 | R1-2112160 | 30 | 0.40% | 99.0% |  |  | 20 | 0.0 | Note 1 |
| Source 7 | R1-2112160 | 30 | 0.40% | 99.0% | Typical HO, Conditional HO |  | 40 | 1.7 |  |
| Source 7 | R1-2112160 | 30 | 0.40% | 99.0% |  |  | 60 | 5.0 |  |
| Source 7 | R1-2112160 | 30 | 0.40% | 99.0% |  |  | 80 | 8.3 |  |
| Source 7 | R1-2112160 | 30 | 0.40% | 99.0% |  |  | 100 | 11.6 |  |
| Source 7 | R1-2112160 | 30 | 0.60% | 99.0% | DAPS |  | 2 | 0.0 | Note 1 |
| Source 7 | R1-2112160 | 30 | 0.60% | 99.0% |  |  | 20 | 0.0 | Note 1 |
| Source 7 | R1-2112160 | 30 | 0.60% | 99.0% | Typical HO, Conditional HO |  | 40 | 2.5 |  |
| Source 7 | R1-2112160 | 30 | 0.60% | 99.0% |  |  | 60 | 7.5 |  |
| Source 7 | R1-2112160 | 30 | 0.60% | 99.0% |  |  | 80 | 12.4 |  |
| Source 7 | R1-2112160 | 30 | 0.60% | 99.0% |  |  | 100 | 17.4 |  |
| Source 7 | R1-2112160 | 30 | 0.80% | 99.0% | DAPS |  | 2 | 0.0 | Note 1 |
| Source 7 | R1-2112160 | 30 | 0.80% | 99.0% |  |  | 20 | 0.0 | Note 1 |
| Source 7 | R1-2112160 | 30 | 0.80% | 99.0% | Typical HO, Conditional HO |  | 40 | 5.0 |  |
| Source 7 | R1-2112160 | 30 | 0.80% | 99.0% |  |  | 60 | 14.9 |  |
| Source 7 | R1-2112160 | 30 | 0.80% | 99.0% |  |  | 80 | 24.8 |  |
| Source 7 | R1-2112160 | 30 | 0.80% | 99.0% |  |  | 100 | 34.7 |  |
| Source 7 | R1-2112160 | 30 | 0.90% | 99.0% | DAPS |  | 2 | 0.0 | Note 1 |
| Source 7 | R1-2112160 | 30 | 0.90% | 99.0% |  |  | 20 | 0.0 | Note 1 |
| Source 7 | R1-2112160 | 30 | 0.90% | 99.0% | Typical HO, Conditional HO |  | 40 | 9.9 |  |
| Source 7 | R1-2112160 | 30 | 0.90% | 99.0% |  |  | 60 | 29.7 |  |
| Source 7 | R1-2112160 | 30 | 0.90% | 99.0% |  |  | 80 | 49.5 |  |
| Source 7 | R1-2112160 | 30 | 0.90% | 99.0% |  |  | 100 | 69.4 |  |
| Source 14 | R1-2112296 | 30 | 0.0% | 99.0% |  |  | 60 | 3 |  |
| Source 14 | R1-2112296 | 30 | 0.5% | 99.0% |  |  | 60 | 5.97 |  |
| Source 14 | R1-2112296 | 30 | 0.9% | 99.0% |  |  | 60 | 29.73 |  |
| Source 13 | R1-2112069 | 30 | 0.0% | 99.0% |  |  | 40 | 1 |  |
| Source 13 | R1-2112069 | 30 | 0.0% | 99.0% |  |  | 42 | 1.2 |  |
| Source 13 | R1-2112069 | 30 | 0.0% | 99.0% |  |  | 44 | 1.4 |  |
| Source 13 | R1-2112069 | 30 | 0.0% | 99.0% |  |  | 46 | 1.6 |  |
| Source 13 | R1-2112069 | 30 | 0.0% | 99.0% |  |  | 48 | 1.8 |  |
| Source 13 | R1-2112069 | 30 | 0.0% | 99.0% |  |  | 50 | 2 |  |
| Source 13 | R1-2112069 | 30 | 0.0% | 99.0% |  |  | 52 | 2.2 |  |
| Source 13 | R1-2112069 | 30 | 0.0% | 99.0% |  |  | 54 | 2.4 |  |
| Source 13 | R1-2112069 | 30 | 0.0% | 99.0% |  |  | 56 | 2.6 |  |
| Source 13 | R1-2112069 | 30 | 0.0% | 99.0% |  |  | 58 | 2.8 |  |
| Source 13 | R1-2112069 | 30 | 0.0% | 99.0% |  |  | 60 | 3 |  |
| Source 13 | R1-2112069 | 30 | 0.0% | 99.0% |  |  | 62 | 3.2 |  |
| Source 13 | R1-2112069 | 30 | 0.0% | 99.0% |  |  | 64 | 3.4 |  |
| Source 13 | R1-2112069 | 30 | 0.0% | 99.0% |  |  | 66 | 3.6 |  |
| Source 13 | R1-2112069 | 30 | 0.0% | 99.0% |  |  | 68 | 3.8 |  |
| Source 13 | R1-2112069 | 30 | 0.0% | 99.0% |  |  | 70 | 4 |  |
| Source 13 | R1-2112069 | 30 | 0.0% | 99.0% |  |  | 72 | 4.2 |  |
| Source 13 | R1-2112069 | 30 | 0.0% | 99.0% |  |  | 74 | 4.4 |  |
| Source 13 | R1-2112069 | 30 | 0.0% | 99.0% |  |  | 76 | 4.6 |  |
| Source 13 | R1-2112069 | 30 | 0.0% | 99.0% |  |  | 78 | 4.8 |  |
| Source 13 | R1-2112069 | 30 | 0.0% | 99.0% |  |  | 80 | 5 |  |
| Source 13 | R1-2112069 | 30 | 0.5% | 99.0% |  |  | 40 | 2.0 |  |
| Source 13 | R1-2112069 | 30 | 0.5% | 99.0% |  |  | 42 | 2.4 |  |
| Source 13 | R1-2112069 | 30 | 0.5% | 99.0% |  |  | 44 | 2.8 |  |
| Source 13 | R1-2112069 | 30 | 0.5% | 99.0% |  |  | 46 | 3.2 |  |
| Source 13 | R1-2112069 | 30 | 0.5% | 99.0% |  |  | 48 | 3.6 |  |
| Source 13 | R1-2112069 | 30 | 0.5% | 99.0% |  |  | 50 | 4.0 |  |
| Source 13 | R1-2112069 | 30 | 0.5% | 99.0% |  |  | 52 | 4.4 |  |
| Source 13 | R1-2112069 | 30 | 0.5% | 99.0% |  |  | 54 | 4.8 |  |
| Source 13 | R1-2112069 | 30 | 0.5% | 99.0% |  |  | 56 | 5.2 |  |
| Source 13 | R1-2112069 | 30 | 0.5% | 99.0% |  |  | 58 | 5.6 |  |
| Source 13 | R1-2112069 | 30 | 0.5% | 99.0% |  |  | 60 | 6.0 |  |
| Source 13 | R1-2112069 | 30 | 0.5% | 99.0% |  |  | 62 | 6.4 |  |
| Source 13 | R1-2112069 | 30 | 0.5% | 99.0% |  |  | 64 | 6.8 |  |
| Source 13 | R1-2112069 | 30 | 0.5% | 99.0% |  |  | 66 | 7.2 |  |
| Source 13 | R1-2112069 | 30 | 0.5% | 99.0% |  |  | 68 | 7.6 |  |
| Source 13 | R1-2112069 | 30 | 0.5% | 99.0% |  |  | 70 | 8.0 |  |
| Source 13 | R1-2112069 | 30 | 0.5% | 99.0% |  |  | 72 | 8.4 |  |
| Source 13 | R1-2112069 | 30 | 0.5% | 99.0% |  |  | 74 | 8.8 |  |
| Source 13 | R1-2112069 | 30 | 0.5% | 99.0% |  |  | 76 | 9.2 |  |
| Source 13 | R1-2112069 | 30 | 0.5% | 99.0% |  |  | 78 | 9.6 |  |
| Source 13 | R1-2112069 | 30 | 0.5% | 99.0% |  |  | 80 | 10.0 |  |
| Source 13 | R1-2112069 | 30 | 0.00% | 99.0% |  |  | 40 | 1.0 |  |
| Source 13 | R1-2112069 | 30 | 0.05% | 99.0% |  |  | 40 | 1.1 |  |
| Source 13 | R1-2112069 | 30 | 0.10% | 99.0% |  |  | 40 | 1.1 |  |
| Source 13 | R1-2112069 | 30 | 0.15% | 99.0% |  |  | 40 | 1.2 |  |
| Source 13 | R1-2112069 | 30 | 0.20% | 99.0% |  |  | 40 | 1.2 |  |
| Source 13 | R1-2112069 | 30 | 0.25% | 99.0% |  |  | 40 | 1.3 |  |
| Source 13 | R1-2112069 | 30 | 0.30% | 99.0% |  |  | 40 | 1.4 |  |
| Source 13 | R1-2112069 | 30 | 0.35% | 99.0% |  |  | 40 | 1.5 |  |
| Source 13 | R1-2112069 | 30 | 0.40% | 99.0% |  |  | 40 | 1.7 |  |
| Source 13 | R1-2112069 | 30 | 0.45% | 99.0% |  |  | 40 | 1.8 |  |
| Source 13 | R1-2112069 | 30 | 0.50% | 99.0% |  |  | 40 | 2.0 |  |
| Source 13 | R1-2112069 | 30 | 0.55% | 99.0% |  |  | 40 | 2.2 |  |
| Source 13 | R1-2112069 | 30 | 0.60% | 99.0% |  |  | 40 | 2.5 |  |
| Source 13 | R1-2112069 | 30 | 0.65% | 99.0% |  |  | 40 | 2.8 |  |
| Source 13 | R1-2112069 | 30 | 0.70% | 99.0% |  |  | 40 | 3.3 |  |
| Source 13 | R1-2112069 | 30 | 0.75% | 99.0% |  |  | 40 | 4.0 |  |
| Source 13 | R1-2112069 | 30 | 0.80% | 99.0% |  |  | 40 | 5.0 |  |
| Source 13 | R1-2112069 | 30 | 0.85% | 99.0% |  |  | 40 | 6.6 |  |
| Source 13 | R1-2112069 | 30 | 0.90% | 99.0% |  |  | 40 | 9.9 |  |
| Source 13 | R1-2112069 | 30 | 0.95% | 99.0% |  |  | 40 | 19.8 |  |
| Note 1: T = 0, Y < PDB |  |  |  |  |  |  |  |  |  |

Table B.2-4. X=95%, PDB=10/15/30ms

| Company | Tdoc | PDB (ms) | PE,op | X | Handover type | Handover case | Y (ms) | T (s) | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | R1-2111046 | 10 | 0.00% | 95.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 0.84 |  |
| Source 18 | R1-2111046 | 10 | 0.00% | 95.0% | Typical HO | FR1-to-FR1 | 62 | 1.04 |  |
| Source 18 | R1-2111046 | 10 | 0.00% | 95.0% | Typical HO | FR1-to-FR1 | 82 | 1.44 |  |
| Source 18 | R1-2111046 | 15 | 0.00% | 95.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 0.74 |  |
| Source 18 | R1-2111046 | 15 | 0.00% | 95.0% | Typical HO | FR1-to-FR1 | 62 | 0.94 |  |
| Source 18 | R1-2111046 | 15 | 0.00% | 95.0% | Typical HO | FR1-to-FR1 | 82 | 1.34 |  |
| Source 18 | R1-2111046 | 30 | 0.00% | 95.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 0.44 |  |
| Source 18 | R1-2111046 | 30 | 0.00% | 95.0% | Typical HO | FR1-to-FR1 | 62 | 0.64 |  |
| Source 18 | R1-2111046 | 30 | 0.00% | 95.0% | Typical HO | FR1-to-FR1 | 82 | 1.04 |  |
| Source 18 | R1-2111046 | 10 | 0.10% | 95.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 0.86 |  |
| Source 18 | R1-2111046 | 10 | 0.10% | 95.0% | Typical HO | FR1-to-FR1 | 62 | 1.06 |  |
| Source 18 | R1-2111046 | 10 | 0.10% | 95.0% | Typical HO | FR1-to-FR1 | 82 | 1.47 |  |
| Source 18 | R1-2111046 | 15 | 0.10% | 95.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 0.75 |  |
| Source 18 | R1-2111046 | 15 | 0.10% | 95.0% | Typical HO | FR1-to-FR1 | 62 | 0.96 |  |
| Source 18 | R1-2111046 | 15 | 0.10% | 95.0% | Typical HO | FR1-to-FR1 | 82 | 1.37 |  |
| Source 18 | R1-2111046 | 30 | 0.10% | 95.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 0.45 |  |
| Source 18 | R1-2111046 | 30 | 0.10% | 95.0% | Typical HO | FR1-to-FR1 | 62 | 0.65 |  |
| Source 18 | R1-2111046 | 30 | 0.10% | 95.0% | Typical HO | FR1-to-FR1 | 82 | 1.06 |  |
| Source 18 | R1-2111046 | 10 | 2.50% | 95.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 1.64 |  |
| Source 18 | R1-2111046 | 10 | 2.50% | 95.0% | Typical HO | FR1-to-FR1 | 62 | 2.03 |  |
| Source 18 | R1-2111046 | 10 | 2.50% | 95.0% | Typical HO | FR1-to-FR1 | 82 | 2.81 |  |
| Source 18 | R1-2111046 | 15 | 2.50% | 95.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 1.44 |  |
| Source 18 | R1-2111046 | 15 | 2.50% | 95.0% | Typical HO | FR1-to-FR1 | 62 | 1.83 |  |
| Source 18 | R1-2111046 | 15 | 2.50% | 95.0% | Typical HO | FR1-to-FR1 | 82 | 2.61 |  |
| Source 18 | R1-2111046 | 30 | 2.50% | 95.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 0.86 |  |
| Source 18 | R1-2111046 | 30 | 2.50% | 95.0% | Typical HO | FR1-to-FR1 | 62 | 1.25 |  |
| Source 18 | R1-2111046 | 30 | 2.50% | 95.0% | Typical HO | FR1-to-FR1 | 82 | 2.03 |  |
| Source 18 | R1-2111046 | 10 | 4.90% | 95.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 39.94 |  |
| Source 18 | R1-2111046 | 10 | 4.90% | 95.0% | Typical HO | FR1-to-FR1 | 62 | 49.45 |  |
| Source 18 | R1-2111046 | 10 | 4.90% | 95.0% | Typical HO | FR1-to-FR1 | 82 | 68.47 |  |
| Source 18 | R1-2111046 | 15 | 4.90% | 95.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 35.19 |  |
| Source 18 | R1-2111046 | 15 | 4.90% | 95.0% | Typical HO | FR1-to-FR1 | 62 | 44.70 |  |
| Source 18 | R1-2111046 | 15 | 4.90% | 95.0% | Typical HO | FR1-to-FR1 | 82 | 63.72 |  |
| Source 18 | R1-2111046 | 30 | 4.90% | 95.0% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 20.92 |  |
| Source 18 | R1-2111046 | 30 | 4.90% | 95.0% | Typical HO | FR1-to-FR1 | 62 | 30.43 |  |
| Source 18 | R1-2111046 | 30 | 4.90% | 95.0% | Typical HO | FR1-to-FR1 | 82 | 49.45 |  |

Table B.2-5. X=99.9%, PDB=10/15/30ms

| Company | Tdoc | PDB (ms) | PE,op | X | Handover type | Handover case | Y (ms) | T (s) | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 18 | R1-2111046 | 10 | 0.00% | 99.9% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 42.00 |  |
| Source 18 | R1-2111046 | 10 | 0.00% | 99.9% | Typical HO | FR1-to-FR1 | 62 | 52.00 |  |
| Source 18 | R1-2111046 | 10 | 0.00% | 99.9% | Typical HO | FR1-to-FR1 | 82 | 72.00 |  |
| Source 18 | R1-2111046 | 15 | 0.00% | 99.9% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 37.00 |  |
| Source 18 | R1-2111046 | 15 | 0.00% | 99.9% | Typical HO | FR1-to-FR1 | 62 | 47.00 |  |
| Source 18 | R1-2111046 | 15 | 0.00% | 99.9% | Typical HO | FR1-to-FR1 | 82 | 67.00 |  |
| Source 18 | R1-2111046 | 30 | 0.00% | 99.9% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 22.00 |  |
| Source 18 | R1-2111046 | 30 | 0.00% | 99.9% | Typical HO | FR1-to-FR1 | 62 | 32.00 |  |
| Source 18 | R1-2111046 | 30 | 0.00% | 99.9% | Typical HO | FR1-to-FR1 | 82 | 52.00 |  |
| Source 18 | R1-2111046 | 10 | 0.01% | 99.9% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 46.66 |  |
| Source 18 | R1-2111046 | 10 | 0.01% | 99.9% | Typical HO | FR1-to-FR1 | 62 | 57.77 |  |
| Source 18 | R1-2111046 | 10 | 0.01% | 99.9% | Typical HO | FR1-to-FR1 | 82 | 79.99 |  |
| Source 18 | R1-2111046 | 15 | 0.01% | 99.9% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 41.11 |  |
| Source 18 | R1-2111046 | 15 | 0.01% | 99.9% | Typical HO | FR1-to-FR1 | 62 | 52.22 |  |
| Source 18 | R1-2111046 | 15 | 0.01% | 99.9% | Typical HO | FR1-to-FR1 | 82 | 74.44 |  |
| Source 18 | R1-2111046 | 30 | 0.01% | 99.9% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 24.44 |  |
| Source 18 | R1-2111046 | 30 | 0.01% | 99.9% | Typical HO | FR1-to-FR1 | 62 | 35.55 |  |
| Source 18 | R1-2111046 | 30 | 0.01% | 99.9% | Typical HO | FR1-to-FR1 | 82 | 57.77 |  |
| Source 18 | R1-2111046 | 10 | 0.05% | 99.9% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 83.96 |  |
| Source 18 | R1-2111046 | 10 | 0.05% | 99.9% | Typical HO | FR1-to-FR1 | 62 | 103.95 |  |
| Source 18 | R1-2111046 | 10 | 0.05% | 99.9% | Typical HO | FR1-to-FR1 | 82 | 143.93 |  |
| Source 18 | R1-2111046 | 15 | 0.05% | 99.9% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 73.96 |  |
| Source 18 | R1-2111046 | 15 | 0.05% | 99.9% | Typical HO | FR1-to-FR1 | 62 | 93.95 |  |
| Source 18 | R1-2111046 | 15 | 0.05% | 99.9% | Typical HO | FR1-to-FR1 | 82 | 133.93 |  |
| Source 18 | R1-2111046 | 30 | 0.05% | 99.9% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 43.98 |  |
| Source 18 | R1-2111046 | 30 | 0.05% | 99.9% | Typical HO | FR1-to-FR1 | 62 | 63.97 |  |
| Source 18 | R1-2111046 | 30 | 0.05% | 99.9% | Typical HO | FR1-to-FR1 | 82 | 103.95 |  |
| Source 18 | R1-2111046 | 10 | 0.09% | 99.9% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 419.62 |  |
| Source 18 | R1-2111046 | 10 | 0.09% | 99.9% | Typical HO | FR1-to-FR1 | 62 | 519.53 |  |
| Source 18 | R1-2111046 | 10 | 0.09% | 99.9% | Typical HO | FR1-to-FR1 | 82 | 719.35 |  |
| Source 18 | R1-2111046 | 15 | 0.09% | 99.9% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 369.67 |  |
| Source 18 | R1-2111046 | 15 | 0.09% | 99.9% | Typical HO | FR1-to-FR1 | 62 | 469.58 |  |
| Source 18 | R1-2111046 | 15 | 0.09% | 99.9% | Typical HO | FR1-to-FR1 | 82 | 669.40 |  |
| Source 18 | R1-2111046 | 30 | 0.09% | 99.9% | Typical HO, Conditional HO | FR1-to-FR1 | 52 | 219.80 |  |
| Source 18 | R1-2111046 | 30 | 0.09% | 99.9% | Typical HO | FR1-to-FR1 | 62 | 319.71 |  |
| Source 18 | R1-2111046 | 30 | 0.09% | 99.9% | Typical HO | FR1-to-FR1 | 82 | 519.53 |  |

Table B.2-6. The evaluation results of PE,op/PER vs. T/Y

| Company | Tdoc | X | PDB (ms) | PE,op/PER | Y (ms) | T (s) | T / Y (Y≠0) | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source 20 | R1-2111351 | 99% | 10 | 0~0.5 | 0~10 | 0~1.99 | 100~199 | Note 1 |
| Source 20 | R1-2111351 |  |  | 0.5~0.8 |  | 0~4.96 | 248.5~496 |  |
| Source 20 | R1-2111351 |  |  | 0.8~0.99 |  | 0~99.01 | 496~9901 |  |
| Source 20 | R1-2111351 |  |  | 0~0.5 | 30~70 | 2.22~11.94 | No analytical formula | Note 2 |
| Source 20 | R1-2111351 |  |  | 0.5~0.8 |  | 4.97~29.76 |  |  |
| Source 20 | R1-2111351 |  |  | 0.8~0.99 |  | 19.82~549.06 |  |  |
| Source 20 | R1-2111351 | 99.5% |  | 0~0.5 | 0~10 | 0~3.99 | 222.11 ~ 399 | Note 1 |
| Source 20 | R1-2111351 |  |  | 0.5~0.8 |  | 0~9.96 | 399~996 |  |
| Source 20 | R1-2111351 |  |  | 0.8~0.99 |  | 0~199.10 | 996~19910 |  |
| Source 20 | R1-2111351 | 95% |  | 0~0.5 |  | 0~0.39 | 22.11~39 |  |
| Source 20 | R1-2111351 |  |  | 0.5~0.8 |  | 0~0.96 | 39~96 |  |
| Source 20 | R1-2111351 |  |  | 0.8~0.99 |  | 0~19.01 | 96~1901 |  |
| Source 20 | R1-2111351 | 90% |  | 0~0.5 |  | 0~0.19 | 11~19} |  |
| Source 20 | R1-2111351 |  |  | 0.5~0.8 |  | 0~0.46 | 19~46 |  |
| Source 20 | R1-2111351 |  |  | 0.8~0.99 |  | 0~9.01 | 46~901 |  |
| Note 1: T = Y * (100%-PE,op) / (100%-X-PE,op), Y<PDB Note 2: T = (Y-PDB) * (100%-PE,op) / (100%-X-PE,op), Y >= PDB |  |  |  |  |  |  |  |  |

# Annex <E> (informative):

# Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-10 | RAN1#103e | R1-2009818 |  |  |  | Skeleton TR | 0.0.1 |
| 2021-10 | RAN1#106b-e | R1-2110678 |  |  |  | Updated TR structure and outcomes endorsed at RAN1#106b-e | 0.1.0 |
| 2021-11 | RAN1#107e | R1-2112812 |  |  |  | Incorporate R1-2112722, R1-2112723, R1-2112809, R1-2112810, R1-2112821 with additional email agreement in email discussion | 0.2.0 |

Note that the PDB defined in this section for XR evaluation purpose only. Its exact definition is different from that of the PDB in 5G system

Note that the exact definition of PER defined for this section is different from that defined in [5].

In details, the described operation may or may not be fully compliant to R17 PDCCH monitoring adaptation scheme which is currently still being discussed in R17 UE PS session as of 107-e.

The loss in UE satisfied rate is said marginal if the DL+UL UE satisfied rate is larger than equal to 80% for a considered power saving scheme when the number of UEs per cell is equal to capacity. This definition applies all other cases and sections.

The loss in UE satisfied rate is said marginal if the DL UE satisfied rate is larger than equal to 80% for a considered power saving scheme when the number of UEs per cell is equal to capacity. This definition applies all other sections.

The loss in UE satisfied rate is said marginal if the UL UE satisfied rate is larger than equal to 80% for a considered power saving scheme when the number of UEs per cell is equal to capacity. This definition applies all other sections.

The loss in UE satisfied rate is said marginal if the DL UE satisfied rate is larger than equal to 80% for a considered power saving scheme when the number of UEs per cell is equal to capacity. This definition applies all other sections.

The loss in UE satisfied rate is said marginal if the UL UE satisfied rate is larger than equal to 80% for a considered power saving scheme when the number of UEs per cell is equal to capacity. This definition applies all other sections

Note that this is not intended to introduce new power class.