| 3GPP TR 38.856 V0.2.0 (2019-10) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Study on local NR positioning in NG-RAN (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	 Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	7

3.3	Abbreviations	7

4	General	7

5 	 Local LMF in NG-RAN	7

5.1	Architecture	7

5.1.1 	 Alternative 1	7

5.1.2 	 Alternative 2	8

5.1.3 	 Alternative 3	9

5.1.4 	 Function supported by LMC [FFS]	9

5.2	Impact on existing protocols and interfaces [FFS]	10

5.2.1	Location Service procedures involving LMC	10

5.2.1.1	RAN Induced Location Request (RI-LR)	10

5.2.1.2	5GC Mobile Terminated Location Request (5GC-MT-LR)	11

5.2.2	Impacts on Signalling protocols and interfaces	12

5.2.2.1	 Signalling between an AMF and gNB/LMC	12

5.2.2.2	 Signalling between a gNB/LMC and UE	13

5.2.2.3	 Signalling between gNBs/LMCs	14

5.2.2.4	 Signalling between gNB-CU and LMC	15

5.3	Coordination and coexistence with LMF in the 5GC	15

6	LCS client in NG-RAN	16

6.1	Architecture	16

6.2	 Impact on existing protocols and interfaces	16

7	Conclusion	16

Annex <X> (informative): Change history	17

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

Introduction

A new SI “Study on local NR positioning in NG-RAN” was approved for Release 16 at the 3GPP TSG RAN #83 meeting in [2]. The study aims to the feasibility and specification impact for supporting local LMF and/or LCS client in NG-RAN.

1		Scope

The present document captures the studies on the feasibility and specification impact of local location management functionality and of the NG RAN acting as LCS client.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP RP-191460:  "Revised SID: Study on local NR positioning in NG-RAN"

[3]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS)"

[4]	3GPP TS 38.305: "NG-RAN; Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[5]	3GPP TS 29.572: “5G System; Location Management Services; Stage 3”

[6]	3GPP TS 36.355: “Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol (LPP)”

[7]	3GPP TS 38.331: “NR; Radio Resource Control (RRC) protocol specification”

[8]	3GPP TS 22.261: “Service requirements for the 5G system; Stage 1”

[9]	3GPP TS 22.104: “Service requirements for cyber-physical control applications in vertical domains; Stage 1”

3	Definitions of terms, symbols and abbreviations

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

LMC: The Location Management Component comprises all or part of the location management function, as defined in clause 4.3.8 of TS 23.273 [3]. The LMC resides in the NG-RAN.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

4	General

5 		Local LMF in NG-RAN

Editor note: Specification impact of Local LMF including location of the LMF, potential new interface(s) (if any), impact on existing protocols, and coordination with the LMF in the 5GC

Editor note: Evaluation section (in case multiple solutions are captured) could be added

5.1	Architecture

5.1.1 		Alternative 1

In architecture alternative 1, the LMC is an internal function of the NG-RAN node. In case of split gNB, the LMC is located in the gNB-CU-CP.

The interface between the LMC and the serving NG-RAN node is internal, and therefore minimizes the latency between the LMC and serving NG-RAN node. Functions of the NLs interface must be specified also for the NG-C interface.

Characteristics:

-	No new interfaces.

-	When UE positioning involves only TRPs within the NG-RAN node, positioning-related signalling is internal to the gNB.

-	To support location continuity in case of handover, LMC relocation to the target NG-RAN node can be enabled via enhancements to the XnAP Handover Preparation procedure. Further study is needed.

Figure 5.1.1-1  Alternative 1 – LMC as internal gNB function

5.1.2 		Alternative 2

In architecture alternative #2, the LMC is a logical node within the split gNB connected to the gNB-CU-CP via the [FFS] interface.

This alternative requires a dedicated interface between the LMC and the serving NG-RAN node. The impacts to the NG-C interface are the same as alternative 1.

Characteristics:

-	New interface between the LMC and the gNB-CU-CP.

-	Allows LMC and gNB-CU-CP to be provided by different vendors.

-	Allows offloading of positioning support from a gNB-CU.

-	To support location continuity in case of handover, further study is needed.

Figure 5.1.2-1 Alternative 2 – LMC as logical node within split gNB

5.1.3 		Alternative 3

In architecture alternative #3, the LMC is a new logical node in the NG-RAN, connected to NG-RAN nodes (gNBs and/or ng-eNBs) via the [FFS] interface.

This alternative requires a dedicated interface between the LMC and the serving NG-RAN node. The impacts to the NG-C interface are the same as alternative 1 and must be supported also by the new interface.

Characteristics:

-	New interface between the LMC and the NG-RAN node.

-	Allows LMC and NG-RAN nodes to be provided by different vendors.

-	Allows a single LMC to support multiple NG-RAN nodes (i.e. avoid introducing LMC in each individual NG-RAN node).

-	Allows offloading of positioning support from a gNB-CU.

-	To support location continuity in case of handover when both source and target NG-RAN nodes are served by the same LMC, LMC relocation is not needed. However, when source and target NG-RAN nodes are served by different LMCs, support for location continuity requires further study.

Figure 5.1.3-1 Alternative 3 – LMC as logical node in NG-RAN

5.1.4 		Function supported by LMC [FFS]

Editor note: At this stage it is not clear if the LMC is able to support all the function of the LMF. The positioning method supported by the LMC is FFS.

The LMC can support all the location request types defined for the LMF in clause 4.1a of TS 23.273 [3].

The LMC should be able to support location service requests from functions internal to the NG-RAN node, this type of location request can be called RAN Induced Location Request (RI-LR). The LMC is ideally suited to handle RI-LR since the necessary operations are internal to the RAN and can therefore be handled locally.

In order to manage the various location services for target UEs, the LMC can be expected to have capabilities similar to an LMF.

Editor note: it is FFS whether service request can come from an internal function of the serving NG-RAN node (e.g. RRM, MDT, etc.);

In addition, the LMC can be expected to:

-	interact with other internal functions of the serving NG-RAN node (e.g. TMF) in order to e.g. obtain position measurements for the UE;

-	interact with neighboring NG-RAN nodes in order to e.g. obtain position measurements for the UE.

5.2	Impact on existing protocols and interfaces [FFS]

5.2.1	Location Service procedures involving LMC

Editor Note: flows are intended to show common operation for all architectures with LMC. Specific architectures may require further details

5.2.1.1	RAN Induced Location Request (RI-LR)

Figure 5.2.1.1-1 illustrates the steps applicable to the UE and NG-RAN node in case of RI-LR involving LMC.

In this scenario, the UE is assumed to be in connected mode prior to step 1. The procedure assumes that the NG-RAN node is authorized to use the LCS service to obtain the target UE location estimation, and that notification and/or verification of location by the UE is not needed, e.g. which may be indicated in UDM privacy subscription for the UE and signaled to the serving gNB by the serving AMF as part of signaling connection establishment.

Figure 5.2.1.1-1: RI-LR involving LMC

The steps of Figure 5.2.1.1-1 are as follows:

1.	Some function in the NG-RAN node (e.g. RRM, MDT) requests some location service (e.g. positioning) for a target UE to the LMC using signaling internal to the NG-RAN node.

EN:	Interactions with the local positioning client are FFS.

EN:	Enhancements to F1AP, if needed (e.g. in case the requesting function is in the DU), are FFS.

2.	The NG-RAN node (LMC) sends an LCS Indication message to the AMF, indicating that UE positioning is being performed locally (LCS session start). The LCS Indication message includes information about the “local” LCS Request that may be useful for the AMF (location management information), e.g. the same or subset of information contained in a “normal” location service request (e.g. MT-LR) such as Location Quality of Service information.

EN:	Whether this step is needed, and under which conditions is FFS; e.g., may involve SA2.

3.	The AMF stores the relevant location management information in the UE location context, to be used for coordination of concurrent LCS requests, e.g. if the AMF receives an MT-LR while there is an active location session at the LMC for the same UE.

EN:	Whether this step is needed, and under which conditions is FFS; e.g., may involve SA2.

EN:	Coordination with the 5GC, if needed, is FFS.

4a.	The LMC may instigate location procedures internal to the NG-RAN node – e.g. to obtain positioning measurements or assistance data. The LMC may also instigate location procedures with neighbor NG-RAN nodes – e.g. to obtain assistance data such as PRS configuration of TPs served by the neighbor NG-RAN node(s).

EN:	Enhancements to XnAP, if needed, are FFS.

4b.	The LMC may instigate location procedures with the UE – e.g. to obtain a location estimate or positioning measurements or to transfer location assistance data to the UE.

5.	The LMC responds to the requesting function in the NG-RAN node and includes any needed results – e.g. success or failure indication and, if requested and obtained, a location estimate for the UE.

6.	If the response in step 5 ends the local LCS session, the NG-RAN node (LMC) sends an LCS Indication message to the AMF indicating that positioning is no longer being performed locally for the target UE (LCS session stop).

EN:	Whether this step is needed, and under which conditions is FFS; e.g., may involve SA2.

5.2.1.2	5GC Mobile Terminated Location Request (5GC-MT-LR)

Figure 5.2.1.2-1 illustrates the steps applicable to the UE and NG-RAN node in case of 5GC-MT-LR involving LMC. In this scenario, the UE is assumed to be in connected mode prior to step 1.

Figure 5.2.1.2-1: 5GC-MT-LR involving LMC

The steps of Figure 5.2.1.2-1 are as follows:

1.	Some LCS entity in the 5GC (e.g. GMLC) requests some location service (e.g. positioning) for a target UE to the serving AMF (see step 4 of Figure 6.1.1-1 in TS.23.273 [3]).

2.	The AMF selects the serving NG-RAN node (LMC) to perform the positioning, based on e.g. knowledge of the NG-RAN node’s location management capabilities, Requested Quality of Service information, etc.

Note 1:	The details of whether to select LMC are SA2 scope, including whether it is part of the LMF Selection function or a new function of the AMF.

3.	The AMF sends a location service request to the NG-RAN node (LMC) via NGAP, instructing it to perform the UE positioning.

4.	Same as steps 4a/4b of the RI-LR procedure in clause 5.2.1.1.

5.	The NG-RAN node (LMC) provides a location service response to the AMF via NGAP and includes any needed results – e.g. success or failure indication and, if requested and obtained, a location estimate for the UE.

6.	The AMF returns a location service response to the 5GC entity of step 1 and includes any needed results – e.g. a location estimate for the UE (see step 10 of Figure 6.1.1-1 in TS.23.273 [3]).

5.2.2	Impacts on Signalling protocols and interfaces

5.2.2.1		Signalling between an AMF and gNB/LMC

The NL1 interface between AMF and LMF supports location requests for a target UE sent from a serving AMF for the target UE to an LMF as specified in TS 29.572 [5] (Nlmf_Location_DetermineLocation Request/Response). The Request operation can include the following parameter (at least one of these parameters must be present):

-	externalClientType, correlationID, amfId, locationQoS, supportedGADShapes, supi, pei, gpsi, ecgi, ncgi, priority, velocityRequested;

and the Response operation may include (where the parameter locationEstimate must be present):

-	locationEstimate, accuracyFulfilmentIndicator, ageOfLocationEstimate, velocityEstimate, civicAddress, positioningDataList, gnssPositioningDataList, ecgi, ncgi, altitude, barometricPressure.

An Nlmf_Location_DetermineLocation Request/Response message could be transported between the serving gNB and serving AMF for a target UE in an NGAP transport container, which could be defined as a new NGAP UL/DL transport message.

This has several advantages, including the following:

-	An AMF can use the same message/operation towards an LMF and LMC.

-	Better functional alignment between an LMC and LMF.

-	Immediate and deferred location requests can be supported in alignment with 5GC location procedures (e.g. MT-LR, MO-LR, NI-LR) defined in TS 23.273 [3] (see also subclause 5.2.1).

Figure 5.2.2.1-1 below shows an example procedure for a basic MT-LR. Steps 5 are the procedures which would be performed if the AMF at Step 4 selects an LMF; steps 6 would be performed if the AMF at step 4 selects a LMC. From an AMF point of view, the same message would be used in both cases; only the transport (container) would be different. Similarly, an LMC would see the same "input/output" data as an LMF.

NOTE:	Whether to reuse AMF/LMF service operations in an NGAP message container or extend/introduce NGAP message with explicit IEs could be decided in a potential work item phase by RAN3.

Figure 5.2.2.1-1: Example of MT-LR Location Service Support using 5GC LMF (Steps 5) and using a NG-RAN LMC (steps 6).

5.2.2.2		Signalling between a gNB/LMC and UE

The LPP protocol TS 36.355 [6] is used for the positioning procedures between an LMF and target UE, as specified in TS 38.305 [4]. For an LMF, LPP messages are carried as transparent PDUs across intermediate network interfaces using the appropriate protocols (e.g., NAS/NGAP over the NG-C interface, NAS/RRC over the Uu interface).

LPP can be reused for the positioning signalling between an LMC and target UE and transported in an RRC message container. The DL and UL Information Transfer messages (TS 38.331 [7]) may be extended to support a non-NAS message container (e.g. as described for Note 1 in clause 5.2.1.1), or a new UL/DL RRC Transfer container message can be defined. The reuse of LPP has the lowest overall impact, which also makes UE positioning procedures agnostic to where the LMF is located (i.e., 5GCN LMF or NG-RAN LMC) (similar to the reuse of NL1 messages (subclause 5.2.2.1)). The Step 6b in Figure 5.2.2.1-1 can then be extended as shown in Figure 5.2.2.2-1.

NOTE:	Whether to reuse LPP in an RRC message container or extend/introduce RRC message with explicit IEs should be decided in the work item phase by RAN2.

- Figure 5.2.2.2-1: LPP signalling between LMC and UE.

For Location Services operation, the Supplementary Services (SS) Protocol (TS 24.080 [xx]) is also used for 5GC Location Services, as specified in TS 23.273 [3]. The MT-LR and MO-LR Services messages are exchanged between an AMF and a UE and can also be used in the same way as currently defined for 5GC LMF location services (see subclause 5.2.1). The Supplementary Services messages for support of periodic and triggered location services are exchanged between a 5GC LMF and UE (TS 23.273 [3]. For an NG-RAN LMC these messages can be transported in the same RRC container message as for the LPP messages and is shown in Figure 5.2.2.2-2.

NOTE:	Whether to reuse SS Protocol in an RRC message container or extend/introduce RRC message with explicit IEs should be decided in the work item phase by RAN2.

- Figure 5.2.2.2-2: Supplementary Services (SS) signalling between LMC and UE.

5.2.2.3		Signalling between gNBs/LMCs

Location procedures between pairs of gNBs/LMCs are required to support one or more of the following functions:

(a)	Request UL measurements for a target UE by one gNB (e.g. a serving gNB with an LMC) from another gNB (TRP).

(b)	Provide assistance data for a target UE by one gNB (e.g. a serving gNB with an LMC) to another gNB to assist UL measurements of the target UE by the other gNB/TRP.

(c)	Request a change in DL PRS broadcast scheduling and configuration by one gNB to a neighbour gNB.

(d)	Request a change in scheduling and resources for broadcast of location information by one gNB to a neighbour gNB.

Items (a)-(d) are the same functionality as required between a 5GC LMF and an NG-RAN Node which can be supported using NRPPa.

5.2.2.4		Signalling between gNB-CU and LMC

The location procedures between a gNB-CU and LMC comprise all location related procedures on NG, Xn, and NR-Uu interfaces:

-	location procedures between AMF and gNB/LMC (using Nlmf_Location_DetermineLocation), as described in subclause 5.2.2.1;

-	location procedures between gNB/LMC and UE (using LPP and SS), as described in subclause 5.2.2.2;

-	location procedures between gNBs/LMCs (using NRPPa), as described in subclause 5.2.2.3.

Essentially, a gNB-CU would forward any location related messages received on NG, Xn and Uu interfaces to the LMC.

NOTE:	Whether to reuse the F1 interface for signaling between the gNB-CU and LMC or introduce a new interface should be decided in the work item phase by RAN3.

5.3	Coordination and coexistence with LMF in the 5GC

Editor Note: Function split between LMC in NG-RAN and LMF in 5GC is FFS

Editor Note: Whether functions supported by LMC can be different per architecture alternative is FFS.

Editor Note: How/who to trigger location request to LMC and whether the location request can come from internal function of NG-RAN node is FFS

In the Rel-15 LCS architecture, the AMF initiates (in case of NI-LR) or receives (in case of MO-LR and MT-LR) a location request, and then selects an LMF to perform the location estimation of the target UE. For LMF selection, the AMF may consider various factors as described in TS 23.273 including Requested Quality of Service information (e.g. LCS accuracy, latency), LMF capabilities, LMF load, LMF location and AMF local configuration. For a given target UE, only one LMF is “in use” to manage the overall coordination and scheduling of resources required for the location of the UE. The AMF also supports LMF re-selection e.g. when the LMF currently “in use” cannot be used for a newly initiated/received location request.

In order to integrate the LMC into the LCS architecture, the AMF must be aware that the NG-RAN node supports LMC and may also need to be aware of some of its capabilities.

NOTE:	Whether this is explicitly signaled by the NG-RAN node to the AMF or configured to the AMF via OAM should be decided in the work item phase.

TS 22.261 [8] defines positioning service levels with corresponding performance requirements. The two positioning service levels requiring the most stringent QoS from a latency perspective are levels 4 and 6 which are only applicable to a 5G enhanced positioning service area, i.e. positioning service levels available in only a subset of the area where 5G is present. TS 22.104 [9] provides typical scenarios which require positioning service levels 4 and 6. It is a working assumption that the AMF shall be able to select the LMC for only certain location requests (e.g. those requiring stringent QoS such as low latency and/or high accuracy corresponding to positioning service levels 4 and 6) while selecting an LMF for all other location requests (e.g. those requiring normal QoS).

In case there are concurrent location requests for the same target UE where at least one requires stringent QoS, there are two possible solutions:

Solution 1:	The concurrent location requests are all handled by a single entity, i.e. the LMC.

-	Description: This solution is consistent with the Rel-16 LMF selection functionality as described in section 5.1 of TS 23.273 where concurrent location requests are preferably handled by the same location management entity, i.e. in Rel-16 a new LCS Request is transferred to the LMF handling an ongoing location session if an LMF ID is available in the UE location context stored in the AMF. In the case of LMC, the UE location context indicates that there is an LMC handling an ongoing location session, and therefore concurrent location requests are transferred by the AMF to the LMC.

-	Potential benefit(s): Alignment with Rel-16 LMF selection principles, enabling the LMC to handle concurrent location requests in a coordinated and efficient way.

-	Potential drawback(s): The LMC may end up handling non-latency-sensitive location requests. In some deployments, it may be desirable to use the LMC only for location requests that require stringent QoS (e.g. low latency and/or high accuracy), while less demanding location requests continue to be served by LMF in the core network. This could be due to the more limited resources (e.g. processing power) at the NG-RAN node. Also, if there is an ongoing location session being handled by an LMF when a concurrent location request (requiring stringent QoS) is triggered for the same target UE, one of the location requests must fail unless a complex mechanism is introduced to move the ongoing location session from LMF to LMC.

Solution 2:	The concurrent location request(s) requiring stringent QoS is handled by the LMC, while the other location request(s) is handled in parallel by an LMF.

-	Description: This solution allows different location requests for the same target UE to be handled concurrently by up to two location management entities: the LMC and an LMF. The AMF provides information about the location request(s) being handled by the LMC to the LMF which is “enhanced” (compared to legacy LMF functionality) to take the information into account when handling concurrent location requests for the same target UE. For example:

a)	The LMF may “fetch” the latest available UE location information from the LMC, if the ongoing location session has appropriate attributes in terms of e.g. accuracy, expected age, etc.; or

b)	The LMF may handle the concurrent location request in an independent way that does not conflict with the LMC. This may make sense if the concurrent location request does not require high accuracy, and thus the LMF is able to handle the request using E-CID that does not conflict with radio configurations being used by the LMC.

-	Potential benefit(s): Enables a deployment to use the LMC only for location requests that require stringent QoS, while less demanding location requests continue to be served by LMFs in the core network.

-	Potential drawbacks(s): Requires some new functionality in the LMF.

Comparing the two solutions, Solution 1 has little to no specification impact and could be acceptable in certain deployment scenarios (e.g. certain private networks and/or certain device types). However, Solution 2 seems to provide more flexibility for diversity of deployments and device types.

6	LCS client in NG-RAN

Editor note: Specification impact of NG RAN acting as LCS client

6.1	Architecture

6.2		Impact on existing protocols and interfaces

7	Conclusion

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | Tdoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-8-26 | RAN3 #105 | R3-184476 |  |  |  | First Skeleton provided | 0.0.0 |
| 2019-8-31 | RAN3 #105 |  |  |  |  | Include the agreed TP in R3-194760 | 0.1.0 |
| 2019-10-24 | RAN3 #105bis |  |  |  |  | Include the agreed TP in R3-196223, R3-196224, R3-196225, R3-196226 | 0.2.0 |