| 3GPP TR 38.894 V0.1.0 (2022-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Requirements for simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	8

4	Background	8

4.1	Background information in Rel-17	8

4.2	WI Objective in Rel-18	9

5	Specific Band combinations	10

5.1	CA_n34A-n41A	10

5.1.1	Status of the band combination	10

5.1.1.1	Operating bands for CA	10

5.1.1.2	Power class of the BC	10

5.1.1.3	∆TIB and ∆RIB	10

5.1.2	MSD analysis for simultaneous Rx/Tx	10

5.1.3	Requirements for simultaneous Rx/Tx	10

5.2	CA_n39A-n41A	11

5.2.1	Status of the band combination	11

5.2.1.1	Operating bands for CA	11

5.2.1.2 Power class of the BC	11

5.2.2	MSD analysis for simultaneous Rx/Tx	11

5.2.3 Requirements for simultaneous Rx/Tx	11

5.3	CA_n40A-n41A	12

5.3.1	Status of the band combination	12

5.3.1.1	Operating bands for CA	12

5.3.1.2	Power class of the BC	13

5.3.1.3	∆TIB and ∆RIB	13

5.3.2	MSD analysis for simultaneous Rx/Tx	13

5.3.3	Requirements for simultaneous Rx/Tx	13

5.4	CA_n7A-n40A	13

5.4.1	Status of the band combination	13

5.4.1.1	Operating bands for CA	13

5.4.1.2	Power class of the BC	14

5.4.1.3	∆TIB and ∆RIB	14

5.4.1.4	Existing MSD requirement	14

5.4.2	MSD analysis for simultaneous Rx/Tx	14

5.4.3	Requirements for simultaneous Rx/Tx	15

5.x	CA_nA-nB	15

5.2.1	Status of the band combination	15

5.2.2	MSD analysis	15

5.2.3	Requirements for simultaneous Rx/Tx	15

Annex A (informative): Change history	15

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document is a technical report for requirements on simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC.

Requested FDD-TDD, TDD-TDD CA, SUL, MR-DC and NR-DC band combinations for evaluation of supporting simultaneous Rx/Tx capability/operation are provided in the table below.

Table 1: Band combinations for evaluation of simultaneous Rx/Tx

| Band combination | Power Class |
| --- | --- |
| CA_n34A-n41A | PC3 |
| CA_n39A-n41A | PC3 |
| CA_n40A-n41A | PC3, PC2 |
| CA_n7A-n40A | PC3 |

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-210890, “New WID on New WID on simultaneous Rx/Tx band combinations for CA, SUL, MR-DC and NR-DC”.

[3]	RP-222648, “New WID: Simultaneous Rx/Tx inter-band combinations for NR CA/DC, NR SUL and LTE/NR DC in Rel-18”.

[4]	3GPP TS 38.101-1: “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

[5]	3GPP TS 38.101-2: “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”.

[6]	3GPP TS 38.101-3: “NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”.

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

A-MPR	Additional Maximum Power Reduction

BCS	Bandwidth Combination Set

CA	Carrier Aggregation

CC	Component Carrier

DC	Dual Connectivity

EIRP	Equivalent Isotropically Radiated Power

EN-DC	E-UTRA/NR DC

EVM	Error Vector Magnitude

FDM	Frequency Division Multiplexing

FR	Frequency Range

ENBW	The aggregated bandwidth of an E-UTRA sub-block and an adjacent NR sub-block

ITS	Intelligent Transportation System

ITU-R	Radiocommunication Sector of the International Telecommunication Union

MBW	Measurement bandwidth defined for the protected band

MPR	Allowed maximum power reduction

MSD	Maximum Sensitivity Degradation

MCG	Master Cell Group

NR	New Radio

NS	Network Signalling

NSA	Non-Standalone, a mode of operation where operation of an other radio is assisted with an other radio

OOB	Out-of-band

OOBE	Out-of-band emission

OTA	Over The Air

PRB	Physical Resource Block

PSCCH	Physical Sidelink Control Channel

PSSCH	Physical Sidelink Shared Channel

RE	Resource Element

REFSENS	Reference Sensitivity

RF	Radio Frequency

Rx	Receiver

SCG	Secondary Cell Group

SCS	Subcarrier spacing

SEM	Spectrum Emission Mask

SL	Sidelink

SUL	Supplementary uplink

TDM	Time Division Multiplex

Tx	Transmitter

UE	User Equipment

UL MIMO	Up Link Multiple Antenna transmission

ULSUP	Uplink sharing from UE perspective

# 4	Background

## 4.1	Background information in Rel-17

Simultaneous Rx/Tx capability for inter-band CA, SUL and EN-DC band combinations were introduced from Rel-15. Specifically, for inter-band CA and EN-DC combination, the capability is used for TDD-TDD and TDD-FDD band combinations. According to the description of the capability, it is conditional mandatory and the condition is described in the field, i.e. indicated in the RAN4 spec which combinations should mandatorily support simultaneous Rx/Tx. For the combinations which have no such indication, the capability is optional, i.e. for UE supporting simultaneous Rx/Tx, the capability should be reported, otherwise, the capability is absent or not reported. Since the capability is important for network scheduling, it should be reported accurately.

In Rel-17, the principles for judging the mandatory capability for a band combination have been discussed, and the cases include:

-	FR1+FR1 FDD-TDD band combination

-	FR1+FR1 TDD-TDD band combination

-	FR1+FR2 FDD-TDD band combination

-	FR1+FR2 TDD-TDD band combination

-	FR2+FR2 TDD-TDD band combination

For some categories, the capability should be checked case by case. Therefore, to facilitate the analysis of specific band combinations, a dedicated WI is needed to avoid the ambiguity for application of the general principles agreed in Rel-17 and the analysis and conclusion should be captured in the TR.

In addition, as the capability is defined for CA, SUL, MR-DC and NR-DC band combinations, and applicability of the corresponding requirements cover different specifications, e.g. TS 38.101-1 and TS 38.101-3, the way to treat simultaneous Rx/Tx capability as well as the requirements should be aligned among the specifications.

The fallback rules are captured as below for reference:

-	Request for additions of band combinations to this WI shall be provided using an agreed template and sent to the 3GPP_TSG_RAN_WG4_NR_BANDS email reflector before a RAN4 Tdoc submission deadline and no new band combinations are allowed to be requested after the deadline except to correct the missing fallback and add more supporting companies for the proposed band combinations.

-	When a proponent requests a new band combination, all the next level fallback configurations shall be listed and recorded in the request template and the status (“New”, “Ongoing”, “Completed”) of all the fallback configurations shall be declared accurately and clearly. For “New” fallback configurations, the proponent shall ensure these fallback configurations are also requested together with the higher order band combination in the same meeting.

-	A band combination configuration can only be considered as completed when all of the fallback configurations are completed and specified in advance or at the same meeting. It is the responsibility of the proponent to ensure the status of all of the fallback mode configurations. Rapporteurs and other companies are encouraged to check the status of all of the fallback configurations once the higher order band combinations are declared as completed.

## 4.2	WI Objective in Rel-18

The objectives of the Simultaneous Rx/Tx inter-band combinations for NR CA/DC, NR SUL and LTE/NR DC WI in Rel-18 are as follows:

1.	Identify feasibility for each requested FDD-TDD and TDD-TDD band combinations for CA, SUL, MR-DC supporting simultaneous Rx/Tx capability/operation based on technical analysis, especially for those with large MSD values.

Note 1: Band combinations considered in this WI have to be introduced first via basket WIs (see 2.3) or completed in previous releases if necessary.

Note 2: Whether the simultaneous Rx-Tx capability could be supported or not depends on the evaluation of MSD for the requested band combinations case by case.

2.	Align the specification treatment of simultaneous Rx/Tx capability for CA, SUL, MR-DC and NR-DC band combinations.

# 5	Specific Band combinations

## 5.1	CA_n34A-n41A

### 5.1.1	Status of the band combination

#### 5.1.1.1	Operating bands for CA

The operating bands for CA_n34-n41 are specified in Table 5.1.1.1-1.

Table 5.1.1.1-1: Inter-band CA operating bands involving FR1

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 8) |
| --- | --- | --- |
| CA_n34-n419 | n34, n41 |  |
| NOTE 8: Applicable when dynamic Tx switching is conducted. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13]. NOTE 9: Only applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. |  |  |

For CA_n34-n41, the current spec only considers scenarios without simultaneous Rx/Tx operation.

#### 5.1.1.2	Power class of the BC

The UE Power Class for uplink CA_n34A-n41A are specified in Table 5.1.1.2-1. The supported power class of the band combination would have impact on the MSD, if any, based on the following study.

Table 5.1.1.2-1 UE Power Class for uplink CA_n34A-n41A

| Uplink CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n34A-n41A |  |  |  |  | 23 | +2/-3 |  |  |

#### 5.1.1.3	∆TIB and ∆RIB

The ΔTIB,c due to CA_n34-n41 are specified in Table 5.1.1.3-1.

Table 5.1.1.3-1: ΔTIB,c due to CA_n34-n41

| Inter-band CA combination | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n34-n41 | n34 | 0.3 |
|  | n41 | 0.3 |

ΔRIB,c due to CA_n34-n41 is set to zero.

It is noted that ΔTIB,c and ΔRIB,c reflect the possible UE architecture to support the CA band combination.

### 5.1.2	MSD analysis for simultaneous Rx/Tx

TBD

### 5.1.3	Requirements for simultaneous Rx/Tx

TBD

## 5.2	CA_n39A-n41A

### 5.2.1	Status of the band combination

#### 5.2.1.1	Operating bands for CA

The operating bands for CA_n39-n41 are specified in Table 5.2.1.1-1.

Table 5.2.1.1-1: Inter-band CA operating bands involving FR1

| NR CA Band | NR Band (Table 5.x-1) |
| --- | --- |
| CA_n39-n41 | n39, n41 |

For CA_n39-n41, the current spec only considers scenarios without simultaneous Rx/Tx.

#### 5.2.1.2 Power class of the BC

The UE Power Class for uplink CA_n39A-n41A are specified in Table 5.2.1.2-1.

Table 5.2.1.2-1 UE Power Class for uplink CA_n39A-n41A

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n39A-n41A | 23 | +2/-3 |

### 5.2.2	MSD analysis for simultaneous Rx/Tx

Table 5.2.2-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n39-n41.

Table 5.2.2-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n39 | 1880 | 1920 | 1880 | 1920 | 3760 | 3840 | 5640 | 5760 | 7520 | 7680 |
| n41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 |

Based on above table, there is no harmonic issue for the band combination of n39 and n41.

Table 5.2.2-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n39 | 1880 | 1920 | 1880 | 1920 | 3760 | 3840 | 5640 | 5760 | 7520 | 7680 |
| n41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 |

Based on above table, there are UL3/DL4 order harmonic mixing issue for the band combination of n39 and n41, i.e.  3rd harmonic of band n41 UL may fall into 4th harmonic of band n39 DL.

### 5.2.3 Requirements for simultaneous Rx/Tx

For CA_n39-n41 supporting simultaneous Rx/Tx, the ΔTIB,c and ΔRIB,c values are given in the tables below.

Table 5.2.3-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n39-n41 | 0.5 | 0.5 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.2.3-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n39-n41 | 0.2 | 0.2 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

The cross band isolation MSD for CA_n39-n41 supporting simultaneous Rx/Tx are defined in table 5.2.3-3.

Table 5.2.3-3: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC3 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n39 | n41 | 1900 | 40 | 15 | 216 (RBstart=0) | 2501 | 10 | 3.3 | >ACLR2 |
| n39 | n41 | 1900 | 40 | 15 | 216 (RBstart=0) | 2546 | 100 | 2.2 | >ACLR2 |
| n41 | n39 | 2546 | 100 | 30 | 270 (RBstart=3) | 1917.5 | 5 | 1.6 | >ACLR2 |
| n41 | n39 | 2546 | 100 | 30 | 270 (RBstart=3) | 1900 | 40 | 1.4 | >ACLR2 |

The harmonic mixing MSD requirements for CA_n39-n41 supporting simultaneous Rx/Tx are FFS.

## 5.3	CA_n40A-n41A

### 5.3.1	Status of the band combination

#### 5.3.1.1	Operating bands for CA

The operating bands for CA_n40-n41 are specified in Table 5.3.1.1-1.

Table 5.3.1.1-1: Inter-band CA operating bands involving FR1

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 8) |
| --- | --- | --- |
| CA_n40-n41 | n40, n41 | No |
| NOTE 8: Applicable when dynamic Tx switching is conducted. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13]. |  |  |

For CA_n40-n41, the current spec only considers scenarios without simultaneous Rx/Tx, which are limited by the applicable requirements of ΔTIB,c and ΔRIB,c.

#### 5.3.1.2	Power class of the BC

The UE Power Class for uplink CA_n40A-n41A are specified in Table 5.3.1.2-1. The supported power class of the band combination would have impact on the MSD, if any, based on the following study.

Table 5.3.1.2-1 UE Power Class for uplink CA_n40A-n41A

| Uplink CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n40A-n41A |  |  | 266,7 | +2/-3 | 23 | +2/-3 |  |  |
| NOTE 6: The UE supports PC3 within NR FDD band, and supports either PC3 or PC2 within NR TDD band. NOTE 7: The UE that supports PC3 within an NR TDD or FDD band and supports PC2 within a second NR TDD band may signal a [HigherPowerLimitCADC] capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2A.4.1.3. The power classes referenced are according to the reported [powerClassPerBand] if indicated or ue-PowerClass otherwise. |  |  |  |  |  |  |  |  |

#### 5.3.1.3	∆TIB and ∆RIB

The ΔTIB,c due to CA_n40-n41 are specified in Table 5.3.1.3-1.

Table 5.3.1.3-1: ΔTIB,c due to CA_n40-n41

| Inter-band CA combination | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n40-n41 | n40 | 0.53 |
|  | n41 | 0.53 |
| NOTE 3: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. |  |  |

ΔRIB,c due to CA_n40-n41 is set to zero.

It is noted that ΔTIB,c and ΔRIB,c reflect the possible UE architecture to support the CA band combination.

### 5.3.2	MSD analysis for simultaneous Rx/Tx

TBD

### 5.3.3	Requirements for simultaneous Rx/Tx

For ΔTIB,c due to CA_n40-n41, 0.5 is applied to UE supporting inter-band carrier aggregation with simultaneous Rx/Tx.

Other requirements are FFS.

## 5.4	CA_n7A-n40A

### 5.4.1	Status of the band combination

#### 5.4.1.1	Operating bands for CA

The operating bands for CA_n7-n40 are specified in Table 5.4.1.1-1.

Table 5.4.1.1-1: Inter-band CA operating bands involving FR1

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 8) |
| --- | --- | --- |
| CA_n7-n40 | n7, n40 |  |
| NOTE 8: Applicable when dynamic Tx switching is conducted. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13]. |  |  |

For CA_n7-n40, the current spec has no limitation on scenarios with or without simultaneous Rx/Tx operation.

#### 5.4.1.2	Power class of the BC

The UE Power Class for uplink CA_n7A-n40A are specified in Table 5.4.1.2-1. The supported power class of the band combination would have impact on the MSD, if any, based on the following study.

Table 5.4.1.2-1 UE Power Class for uplink CA_n7A-n40A

| Uplink CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n7A-n40A |  |  |  |  | 23 | +2/-3 |  |  |

#### 5.4.1.3	∆TIB and ∆RIB

The ΔTIB,c due to CA_n7-n40 are specified in Table 5.4.1.3-1.

Table 5.4.1.3-1: ΔTIB,c due to CA_n7-n40

| Inter-band CA combination | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n7-n40 | n7 | 0.5 |
|  | n40 | 0.6 |

The ΔRIB,c due to CA_n7-n40 are specified in Table 5.4.1.3-2.

Table 5.4.1.3-2: ΔRIB,c due to CA_n7-n40

| Inter-band CA combination | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n7-n40 | n40 | 0.5 |

It is noted that ΔTIB,c and ΔRIB,c reflect the possible UE architecture to support the CA band combination.

#### 5.4.1.4	Existing MSD requirement

MSD values due to crossband isolation specified in Rel-17 are in brackets, which should be further studied.

Table 5.4.1.4-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC3 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n7 | 2350 | 100 | 30 | 270 (RBstart=3) | 2622.5 | 5 | [21.9] | >ACLR2 |
| n40 | n7 | 2350 | 100 | 30 | 270 (RBstart=3) | 2645 | 50 | [13.5] | >ACLR2 |

### 5.4.2	MSD analysis for simultaneous Rx/Tx

TBD

### 5.4.3	Requirements for simultaneous Rx/Tx

TBD

## 5.x	CA_nA-nB

For reference of the TP.

### 5.2.1	Status of the band combination

### 5.2.2	MSD analysis

### 5.2.3	Requirements for simultaneous Rx/Tx

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-10 | RAN4#104-bis-e | R4-22xxxxx |  |  |  | TR skeleton | 0.0.1 |
| 2022-11 | RAN4#105 | R4-2219415 |  |  |  | Implemented TP’s from RAN4#105 R4-2220484 TP for TR 38.894 to include simultaneous Rx/Tx CA_n39-n41, ZTE R4-2220485 TP for TR38.894: CA_n39A-n41A ,CA_n34A-n41A, CA_n40A-n41A, CA_n7A-n40A Huawei, HiSilicon TR 38.894 | 0.1.0 |