ETSI/STC/SMG1 - Ad-hoc	SMG1 SC TDoc 4r3/94

Stockholm, Sweden

7 - 9 June 1994

Title:	Support of Optimal Routing

Source:	SMG1 Ad-hoc Group on Optimal Routing

Support of Optimal Routing (SOR)

Service Description, Stage 1

Version 0.0.2

6 June, 1994

Editorial Note:

This draft standard forms the current achievements in SMG1 concerning the specification of all necessary means for the support of Optimal Routing.

A Footnote is inserted, in Italics, upon the following occasions:

-	Something is not covered by this first stage of SOR, but might be addressed at a later stage.

-	Other STC's are asked for guidance.

Table of contents

<to be inserted>

1	Scope

2	References

<to be inserted>

3	Definitions and abbreviations

<to be inserted>

4	Description of Optimal Routing

Optimal Routing is a network feature, by which calls directed to a mobile subscriber, who has roamed away from his HPLMN, are routed directly (instead of via his HPLMN) to the mobile subscriber's actual location, or the forwarded-to destination.

Note:	The principles for Optimal Routing may also apply within the HPLMN, but this is outside the scope of this standard.

Optimal routing shall be used only if a reduction in costs is obtained and provided acceptable quality of service can be maintained.

Reduced costs due to the optimized routing of a given call may reveal that the subscriber is not at the location expected. If the subscriber wishes to preserve his privacy an appropriate means must be provided. The supplementary service Optimal Routing Privacy (ORP) specifies the network and MS means necessary for maintaining the privacy of the subscriber. The invocation of ORP may not hinder the optimal routing of given call, [but the service appearance (e.g. without revealing the location of other user) to the involved parties shall be as without an optimized route].

This specification does not address the following:

-	The portion of Fixed Network Originated Calls until they enter the PLMN are excluded from the call scenarios for the reason that any solutions which aim at the optimization of call routing within the Fixed Network, would need an active support from the Fixed Network by appropriate signalling means. However, GSM operators may be in the position to introduce optimization schemes on a mutual base with the Fixed Network operator, including the fixed network portion of the call. Such additional means are outside the scope of this standard.

-	All call scenarios leading to Multiple Call Forwardings are outside the scope of this standard. Generic solutions for optimal routing described within this standard may be supplemented with specific amendments for Multiple Call Forwardings at a later stage.

-	There is no need for optimization of the routing of Fixed Network terminated calls, because the physical address of a Fixed Network terminating line cannot differ from its logical address. Optimization of calls to UPT subscribers, residing in Fixed Networks are outside the scope of this standard .

-	Support of Optimal Routing in non-GSM mobile networks is not subject of this standard, but might be possible on mutual base among GSM and those non-GSM mobile network operators.

The implementation of the network feature Optimal Routing shall be optional.

In some cases supplementary services may have to be modified to interact correctly with Optimal Routing, e.g. Call Barring.

This Stage 1 description

-	compiles the basic service requirements for Optimal Routing, based on the principles elaborated by GSM MoU,

-	collects the necessary amendments for Supplementary Services in order to cater for Optimal Routing,

-	collects additional specifying text for call set-up procedures and mobile station features,

-	contains the Optimal Routing Privacy Supplementary Service specification.

5	Functional Requirements

5.1	General

The serving network shall provide means for an optimization of a given call from the calling party to the called party and/or the forwarded-to party before the call is actually completed, no matter whether the called party is roaming, the called party has activated Call Forwarding etc.

If the serving network supports SOR, the functional requirements as modifications to current Supplementary Services have to be supported.

5.2	Normal Procedures

5.2.1	Provision

SOR is provided for all Telecommunication Services, except Emergency Calls . The network feature is offered to all home and visiting subscribers served by the network.

5.2.2	Withdrawal

Not applicable.

5.2.3	Registration

Not applicable.

5.2.4	Erasure

Not applicable.

5.2.5	Activation

Not applicable.

5.2.6	Deactivation

Not applicable.

5.2.7	Invocation

SOR is invoked automatically by the network at every call set-up.

5.2.8	Normal operation with successful outcome

The route of a call from calling party to the called party and/or the forwarded-to party is optimized.

A given call is only optimized if the charge of the optimized route (i.e. the roaming leg or the call forwarding leg) is not greater then the charge for the call towards the HPLMN of the called party.

The called or calling party may choose to preserve his privacy, as OR may reveal that the subscriber is not at the location expected. This may be registered at the network using the OR Privacy Supplementary Service. OR Privacy does not necessarily prevent the call from being optimally routed. The OR privacy must be activated before call set-up to indicate that privacy must be maintained.

[If optimal routing is applied and the subscriber will benefit from reduced charges this should be indicated towards the mobile.]

5.2.9	Interrogation

Not applicable.

5.2.10	Charging requirements

In case of SOR is invoked and the called party has subscribed to AoC, AoC parameters for the called party have to be set to zero.

5.3	Exceptional procedures or unsuccessful outcome

5.3.1	Roaming into networks not supporting SOR

If the VPLMN of the called party does not support SOR and the called party has subscribed to AoC and/or ORP, the serving network of the calling party cannot invoke SOR.

As a minimum requirement for the VPLMN of the called party the VPLMN must be able to generate call records for the terminating leg indicating Optimal Routing, otherwise Optimal Routing cannot take place.

5.3.2	Non-support of SOR in the HPLMN

If the HPLMN of the called party does not support SOR, SOR cannot be invoked and the route of the call cannot be optimized (the call has to be routed via the HPLMN of the called party).

6	Adaptations for Support of Optimal Routing

6.01	Requirements for call set-up procedures (GSM 02.01, ...)

>Nicky Hodgson<

No additional requirements.

6.07	Mobile Station Features (GSM 02.07, ...)

>Nicky Hodgson<

No additional requirements.

6.82	Call Forwarding (GSM 02.82, ...)

>Rola Nevoux<

6.82.1	Unconditional Call Forwarding

If Unconditional Call Forwarding is invoked the optimal routing procedures for basic calls shall apply.

6.82.2	Conditional Call Forwarding

If Conditional Call Forwarding is invoked the optimal routing procedures for basic calls shall apply.

6.84	Multi Party (GSM 02.84, ...)

>Erwin Postmann<

<to be inserted, see SMG1 SC TDoc 5/94, to be discussed at next SMG1 meeting>

6.86	Advice of Charge (GSM 02.86, 02.24...)

>Nicky Hodgson<

AoC shall reflect any reduction in charges achieved by OR, based on the location of the calling and called party rather then the dialled number.

Changes to 02.24 see Annex 02.24.

6.86	Call Barring (GSM 02.88)

>Günter Schmidt, DeTeMobil<

It is assumed that Barring of outgoing and incoming international calls is used by the subscriber for cost control reasons.

Under this assumption the following shall happen:

If SOR is invoked, the Barring Programs will be applied to the actual location of the called and/or calling party (and not to the called and/or calling MSISDN).

Changes to 02.88 see Annex 02.88.

<interaction with CF: to be inserted, see SMG1 SC TDoc 11/94, to be discussed at next SMG1 meeting>

6.91	Call Transfer (GSM 02.91, ...)

>Erwin Postmann<

<to be inserted, see SMG1 SC TDoc 5/94, to be discussed at next SMG1 meeting>

6.93	CCBS (GSM 02.93, ...)

>Erwin Postmann<

<to be inserted,see SMG1 SC TDoc 5/94, to be discussed at next SMG1 meeting>

6.xx	CAMEL (Customized Application for Mobile Network Enhanced Logic, 02.78, ...)

>Lena Larsson<

<to be inserted, see SMG1 SC TDoc 13/94r1, to be discussed at next SMG1 meeting>

6.xx	Optimal Routing Privacy (GSM 02.xx, ...)

>Nicky Hodgson<

This is a new supplementary service, see Annex A. Currently this service is focussed on Optimal Routing, but maybe elaborated later on towards further applications in order to cater for location privacy in a more general way.

<Annex B of 02.30 has to be supplemented by the MMI for the ORP>

<Annex A has not been discussed, to be discussed at next SMG1 meeting>

Annex 02.24

2	Introduction

The principle of this service is that the mobile station shall be capable of indicating the cost of a call in home units as a basic service. This is the Advice of Charge (AOC).

The ability for the MS to perform further processing on AOC is not precluded. If the subscriber wishes, the MS shall indicate the value of CCM, ACM, ACMM in the currency she has indicated using the PUCT.

At the charging point, the MS is informed of the charging rate.

The MS shall then use its independent internal clock to time the call from the charging point to the end of call i.e. the chargeable duration (CDUR) is measured by the MS. In the case of multiple calls this applies separately for each call (see section 4.3 l)).

By using the calculations described in section 4, the MS shall be able to derive the number of home units used.

Due to the independence of the calculated value,  an exact one-to-one relation with the bill cannot be guaranteed. This discrepancy is due to the short delays in signalling between the MS and the network, e.g. transmission of charging point and end of call signals.

In deriving this specification the following principles are assumed:

i)	For mobile originated calls, the mobile user pays for the connection to the dialled number, as per the published tariff of the Local PLMN, plus a mark-up defined by the HPLMN to cover additional administration costs, when roaming.

It is assumed that the MS subscriber will NOT be charged for the forwarded leg if the dialled number has set call forwarding. If additional charging is required for this forwarded leg, then it is assumed  that such charging will be applied only to the called party. This follows current telecommunications practice.

ii)	For mobile terminated calls, any charge set for incoming calls is that based on the tariff as published by the HPLMN.

The tariff as published for the roaming extension charges is assumed to be time and date invariant. For the avoidance of doubt, this assumption does not preclude routine tariff changes.

iii)	Charge rates for calls originating within a PLMN vary depending upon, for example, location, destination, service, time of day, type of day and any mark ups.

iv)	The units indicated in the advice of charge are always given in terms of Home units, the value of which is defined and published by the HPLMN, regardless of the PLMN in which they were incurred or the call direction or the type of call including supplementary service and data calls.

v)	AOC shall reflect any reduction in charges achieved by Optimal Routing based on the location of the called and calling parties, rather than the dialled number.

Annex 02.88

Extract from 02.88 Version 4.3.0:

1.2	DESCRIPTION.v.:1.2	DESCRIPTION;

1.2.1	Description

The mobile subscriber may determine by subscription of a set of one or more unique barring programs what kind of outgoing calls shall be barred.

The following barring programs are defined:

1:	Barring of all outgoing calls

2:	Barring of outgoing international calls

3:	Barring of outgoing international calls except those directed to the home PLMN country

The PLMN will ensure that only one of the barring programs is active  per basic service group. The activation of one specific barring program will override an already active one (i.e. the old one will be permanently deactivated).

Description of contents of barring programs:

Barring of all outgoing calls:

With this barring program there are no outgoing set-up possibilities, except emergency calls.

Barring of outgoing international calls:

Outgoing call set-up possibilities exist only to subscribers (home subscribers and roaming subscribers) of served by the PLMN(s) and to subscribers of the fixed network(s) of the country where the served mobile subscriber is presently located. So the present PLMN may be the home PLMN or a visited PLMN, respectively the fixed network may be that of the home PLMN country or that of a visited PLMN country.

Note:	This definition implies that mobile subscribers, subscribed to a PLMN in the country in which the served mobile subscriber roams, can be called only when located in/served by a PLMN in the same country.irrespective where they roam. On the other hand, mobile subscribers, subscribed to any PLMN in another country than the one in which the served  mobile subscriber roams, cannot can be called even if they roam in the same a PLMN in the same country area as the served mobile subscriber.

Barring of all outgoing international calls except those directed to the home PLMN country:

Outgoing call set-up possibilities exist only to subscribers (home subscribers and roaming subscribers) served by of the PLMN(s) and to subscribers of the fixed network(s) of the country where the served mobile subscriber is presently located or to mobile subscribers (home subscribers and roaming subscribers) served by a PLMN in of the home PLMN country of the served mobile subscriber and to subscribers of the  fixed network(s) in the home PLMN country. So the present PLMN may be the home PLMN or a visited PLMN, respectively the fixed network may be that of the home PLMN country or that of a visited PLMN country.

Note:	This definition implies that mobile subscribers, subscribed to a PLMN in the country in which the served mobile subscriber roams or mobile subscribers subscribed to any PLMN in the same country as the served mobile subscriber home PLMN can be called only when located in/served by a PLMN in either of those two countries.irrespective where they roam. On the other hand, mobile subscribers, subscribed to any PLMN in another country than the one in which the served  mobile subscriber roams and not belonging to the same country as the served mobile subscriber, cannot can be called even if they roam in the same a PLMN in the same country area as the served mobile subscriber or if they roam in the home PLMN country of the served mobile subscriber.

2	BARRING OF INCOMING CALLS.v.2	BARRING OF INCOMING CALLS;

2.1	DEFINITION.v.:2.1	DEFINITION;

This service makes it possible for a mobile subscriber to have barring of certain categories of incoming calls according to a barring program which is selected from a set of one or more barring programs chosen at  provision time and is valid for all incoming calls, or just those associated with a specific basic service group.

The ability of the served mobile subscriber to set-up outgoing calls remains unaffected.

2.2	DESCRIPTION.v.:2.2	DESCRIPTION;

2.2.1	Description

The mobile subscriber may determine by subscription of a set of one or more  barring programs what  kind  of incoming  calls shall be barred.

The following barring programs are defined:

1:	Barring of all incoming calls

2:	Barring of incoming calls when roaming outside the home PLMN country.

The PLMN will ensure that only one of the barring programs is active per basic service group. The activation of one specific barring program will override an already active one (i.e. the old one will be permanently deactivated).

Description of contents of barring programs:

Barring of all incoming calls:

With this barring program there will be no set-up of incoming calls to the served subscriber.

Barring of incoming calls when roaming outside the home PLMN country:

This barring programme is not applicable for the initial phase of the OR feature.

[ For subsequent phases of the OR feature the following shall apply:

With this barring program, calls which originate outside the PLMN in which the served subscriber is roaming and calls which do not originate in the fixed network(s) of the country where the mobile subscriber is presently located and which are terminated for the served subscriber will be barred if the subscriber is roaming outside the home PLMN country (ie. the program is active and operative). The ability to receive calls in the home PLMN country remains unaffected (ie. the program is active and quiescent).]

Annex A

1	Optimal Routing Privacy (ORP)

1.1.	Definition

The ORP supplementary service provides the subscriber with the ability to preserve his privacy by not revealing the subscriber's location through the charging. The invocation of ORP may not hinder optimal routing of a given call, but the , thus the call should be considered as a non-optimally routed call for the pruposes of interaction with other supplementary services.

call charges will be based on routing via the HPLMN.

1.2.	Description

1.2.1.	Description

This supplementary service provides for the ability to ensure privacy of location is maintained whilst roaming. Optimal routing may still occur but the benefits may not be passed to the subscriber and the charges will not indicate the location of the charged party.

1.2.2.	Applicability to telecommunication services

The ORP supplementary service is applicable to all telecommunication services, except for the Short Message Service (?).

1.3.	Normal procedures with successful outcome

1.3.1.	Provision

This Supplementary Service will be provisioned after prearrangement with the service provider.

1.3.2.	Withdrawal

The ORP supplementary service shall be withdrawn at the customer's request or for administrative reasons.

1.3.5.	Activation

The ORP supplementary service is activated via an MMI procedure in the MS, before call set-up, or by the service provider. The user will be informed of the success of the activation.

1.3.6.	Deactivation

The ORP supplementary service is deactivated as a result of an MMI procedure in the MS, or by the service provider. The user will be informed of the success of the deactivation.

1.3.7.	Invocation

The network shall automatically invoke the ORP supplementary service in the call set-up phase.

1.3.8.	Normal operation with successful outcome

Either the called or calling party may have ORP invoked. The fact that ORP has been invoked will not be shown to the other party involved in the call, nor will the fact that optimal routing may occur.

Calling Party has ORP and optimal routing used:

The calling party may pay optimal routing charges. 	     	The called party will pay as if routed via HPLMN.

Called Party has ORP and optimal routing used:

The calling party will pay cost of dialled number.

The called party may pay optimal routing charges.

Calling and Called Party has ORP and optimal routing used:

Both parties will pay as if routed via HPLMN.

1.3.11.	Interrogation

Status check

The mobile subscriber can request the status of the supplementary service and be informed if the service is provided to her.

1.4.	Exceptional procedures or unsuccessful outcome

If the visited network does not support OR, then calls will be charged as though routed via the HPLMN. If the visited network does not support ORP, then privacy cannot be maintained.

1.5.	Alternate procedures

None identified.

1.6.	Interactions with other supplementary services

The invocation of ORP may not hinder optimal routing of a given call, but the call charges will be based on routing via the HPLMN, thus the call should be considered as a non-optimally routed call for the purposes of interaction with other supplementary services.

Invocation of this supplementary service does not prevent the use of any other Supplementary Service. These will be invoked in the same manner as if optimal routing privacy was not available.

1.7.	Interworking considerations

Optimal routing and therefore ORP is available for mobile to mobile calls, and mobile to fixed calls.

History

Document history

It might be worthwhile to address this matter as well, especially because of the fact that the support of UPT Phase1/Phase 2 is subject of the GSM Phase 2+ work programme. Undoubtedly in this respect a close co-operation among NA7 (UPT), NA6 (IN) and probably some of the SPS groups is necessary.

SMG4 is asked for guidance: Applicability of SOR to SMS, PAD and Dedicated PAD Access ?

TG GPRS is asked for guidance: Applicability of SOR to GPRS services ?

At a later stage the total amount of costs, including all legs of the call, shall be taken into account for the calculation of the optimal rout of the call. This may imply that the HPLMN of the called party has to provide cost estimates for the roaming leg towards the serving network of the calling party.

This indication is not required for the following reasons: There is no unambiguous indication to the user, an exact indication is given by AoC, if an indication is not received we may discourage users to place a or accept a call because they may think it is very costly, it is difficult to decide at what cost saving the indication should be given,