3GPP TR 21.917 V0.2.0 (2022-01)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Release 17 Description;

Summary of Rel-17 Work Items

(Release 17)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

Description, Summary, Release 17

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	 Abbreviations	7

4	Rel-17 Executive Summary	8

5	5G Services to "verticals"	8

5.1	Network improvements for verticals	8

5.2	Location and positionning	8

5.3	Railways	8

5.4	Support of Non-Public and Private Networks	8

5.5	Industrial IoT	9

5.6	Media production	9

5.7	Other services to verticals	9

6	5G access via Satellite/Non-Terrestrial (NTN) link	10

7	V2V/D2D/Sidelink related	10

8	Network architecture optimisations	11

8.1	Edge computing	11

8.2	Slicing	11

8.3	Access Traffic Steering, Switch and Splitting support in the 5G system architecture; Phase 2	12

8.4	Self-Organizing (SON)/Autonomous Network	12

8.5	Minimization of service Interruption	12

9	Multicast-Broadcast Services (MBS) and Mission Critical (MC)	13

10	Multi-(U)SIM	13

11	MSG in 5G	13

12	Drone/UAS/UAV/EAV identification	14

13	Authentication and key management for applications based on 3GPP credential in 5G	14

14	Multimedia Priority Service (MPS) Phase 2	14

15	NR enhancements	15

16	Other NR-only improvements	16

17	NR Carrier Aggregation	18

17.1	NR intra band Carrier Aggregation (itrabCA)	18

17.2	NR inter band Carrier Aggregation (iterbCA)	18

18	NR and LTE	18

19	NR new/modified bands	20

20	LTE new/modified bands	21

21	LTE inter-band Carrier Aggregation (LTE iterbCA)	22

22	Other Release 17 Features	22

Annex A: List of expected contributions	26

Annex B: Template for contributing	38

Annex C: Process to get further information	39

C.1	General	39

C.2	Direct method	40

C.3	Step by step method	40

Annex D: Change history	43

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document provides a summary of each and every 3GPP Release 17 Feature, and more generally of all Work Items for which a summary has been agreed to be provided.

These summaries are based on the inputs issued by the Work Item Rapporteurs, sometimes modified by the Work Plan manager to ensure overall consistency of this document. The original inputs can be retrieved as temporary document (tdoc). The original tdoc number is provided just below the table of each clause.

The present document presents the "initial state" of the Features introduced in Release 17, i.e. as they are by the time of publication of this document. Each Feature is subject to be later modified or enhanced, over several years, by the means of Change Requests (CRs). To further outline a feature at a given time, it is recommended to retrieve all the CRs which relate to the given Feature, as explained in Annex C. As a first approach, the list of all the Change Requests related to a Work Item can be retrieved by selecting "TSG Status = Approved" in the Work Item’s page of the 3GPP Portal at:

https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem= [6-digit identifier of the Work Item, separated by a coma if several Work Items]

E.g. for “NR support for high speed train scenario”: the table at the beginning of its section indicates that the corresponding Work Items have the numbers 840192 and 840292. So, all the CRs related to this Feature are retrieved by selecting "TSG Status = Approved" in https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=840192,840292

Editor’s note: INPUTS ARE EXPECTED from rapporteurs. The list of expected contributors is provided at the beginning of each section. 
It also appears in “Annex B: List of contributors”, in the cell highlighted in yellow. Annex A provides the expected rapporteur in term of Working Group. E.g. if “SA2” is indicated, it means: “The SA2 WID rapporteur is expected to provide the summary for the full feature”.
The name of the rapporteur is provided in “Annex C: Full list of Rel-16 Work Items”, in the column “WI_rapporteur_e_mail”.

Editor’s note: to the contributors: 
The template to be used for the contribution is provided in “Annex A: Template for contributing”. The input type in 3GU is “WI Summary”.
The target audience of this document are readers inside and outside the 3GPP community. Contributions should be written as to be understandable by non-specialist. 
The summary should reflect the actual outcome of the Feature - and not the initial intentions given in the WID.
An exhaustive list of all the CRs and TS/TRs is not to be provided in the summary (instead, the procedure on how to retrieve this information is given in section 4). Only the essential CRs and TS/TRs can be mentioned in the Reference. 
When the usual Stage 1/Stage 2/Stage 3 process is not followed -and in particular if a full stage is missing- explain why.

# 1	Scope

The present document provides a summary of each Release 15 Feature or, whenever needed, of each significant Work Item.

The information provided in the present document is limited to an overview of each Feature, explaining briefly its purpose and the main lines of the system's behaviour to execute the Feature.

More information is available by consulting the 3GPP Ultimate web site, as explained in "Annex C: Process to get further information".

The present document presents the "initial state" of the Features introduced in Release 15, i.e. as they are by the time of publication of the present document. Each Feature is subject to be later modified or enhanced, over several years, by the means of Change Requests (CRs). It is therefore recommended to retrieve all the CRs which relate to the given Feature, as explained in Annex C, to further outline a feature at a given time.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	TR 21.905: "Vocabulary for 3GPP Specifications".

NOTE:	Due to the specificity of the present document, consisting in a collection of independent summaries, the references are given at the end of each clause rather than in this clause.

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

## 3.2	Symbols

Void.

## 3.3		Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1]. Abbreviations specific to a given clause are provided in the clause they appear.

KPI	Key Performance Indicator

Rel	3GPP Release

# 4	Rel-17 Executive Summary

Editor's note: To be provided later.

# 5	5G Services to "verticals"

## 5.1	Network and application enablement for verticals

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 920049 | Enhanced Service Enabler Architecture Layer for Verticals | eSEAL |  | SP-200987 | Basavaraj (Basu) Pattan, Samsung |
| 900024 | Stage 2 of eSEAL | eSEAL | S6 | SP-200987 | Basavaraj (Basu) Pattan, Samsung |
| 920006 | CT1 aspects of eSEAL | eSEAL | C1 | CP-212098 | Sapan Shah (Samsung Electronics) |
| 920050 | CT3 aspects of eSEAL | eSEAL | C3 | CP-212098 | Sapan Shah (Samsung Electronics) |
| 920058 | Rel-17 Enhancements of 3GPP Northbound Interfaces and Application Layer APIs | NBI17 |  | CP-211197 | Yali Yan, Huawei |
| 920013 | CT1 aspects of NBI17 | NBI17 | C1 | CP-211197 | Yali Yan, Huawei |
| 920053 | CT3 aspects of NBI17 | NBI17 | C3 | CP-211197 | Yali Yan, Huawei |
| 920054 | CT4 aspects of NBI17 | NBI17 | C4 | CP-211197 | Yali Yan, Huawei |
| 920012 | Enhancement of 5G PCC related services in Rel-17 | en5GPccSer17 | C3 | CP-211193 | Xiaoyun Zhou, Huawei |

Summary based on the input provided by XXX in xP-21xxxx.

## 5.2	Location and positionning

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 910052 | Enhancement to the 5GC LoCation Services-Phase 2 | 5G_eLCS_ph2 |  | SP-200082 | Ming Ai, CATT |
| 870001 | Stage 2 of 5G_eLCS_ph2 | 5G_eLCS_ph2 | S2 | SP-200082 | Ming Ai, CATT |
| 910006 | CT aspects of 5G_eLCS_ph2 | 5G_eLCS_ph2 | ct | CP-211090 | Chenxi Bao, CATT |
| 910053 | CT1 aspects of 5G_eLCS_ph2 | 5G_eLCS_ph2 | C1 | CP-211090 | Chenxi Bao, CATT |
| 910054 | CT3 aspects of 5G_eLCS_ph2 | 5G_eLCS_ph2 | C3 | CP-211090 | Chenxi Bao, CATT |
| 910055 | CT4 aspects of 5G_eLCS_ph2 | 5G_eLCS_ph2 | C4 | CP-211090 | Chenxi Bao, CATT |
| 900060 | RAN aspects of NR positioning enhancements | NR_pos_enh |  | RP-210903 | Intel |
| 860034 | Study on NR positioning enhancements | FS_NR_pos_enh | R1 | RP-202094 | CATT |
| 900160 | Core part: NR positioning enhancements | NR_pos_enh-Core | R1 | RP-210903 | Intel |
| 900260 | Perf. part: NR positioning enhancements | NR_pos_enh-Perf | R4 | RP-210903 | Intel |

Summary based on the input provided by XXX in xP-21xxxx.

## 5.3	Railways

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 840042 | Complete Gap Analysis for Railways Mobile Communication System | MONASTERYEND | S1 | SP-190312 | Gach, Guillaume, UIC |
| 890035 | Enhancements to Application Architecture for the Mobile Communication System for Railways Phase 2 | eMONASTERY2 |  | SP-191104 | Oettl, Martin, Nokia |
| 840037 | Stage 2 of eMONASTERY2 | eMONASTERY2 | S6 | SP-191104 | Oettl, Martin, Nokia |
| 890063 | Stage 3 of eMONASTERY2 | eMONASTERY2 | C1 | CP-202256 | Gkatzikis, Lazaros, Nokia(Nokia) |
| 840043 | Study on Future Railway Mobile Communication System3 | FS_FRMCS3 | S1 | SP-200578 | Gach, Guillaume, UIC, |

Summary based on the input provided by XXX in xP-21xxxx.

See also "Introduction of 900MHz NR band for Europe for Rail Mobile Radio (RMR)" and "Introduction of 1900MHz NR TDD band for Europe for Rail Mobile Radio (RMR)" under "NR enhancements" section

## 5.4	Support of Non-Public and Private Networks

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 840024 | Study on enhanced support of Non-Public Networks | FS_eNPN | S2 | SP-200094 | Peter Hedman |
| 910065 | Enhanced support of Non-Public Networks | eNPN |  | SP-200980 | Hedman, Peter, Ericsson |
| 900015 | Stage 2 for eNPN | eNPN | S2 | SP-200980 | Hedman, Peter, Ericsson |
| 910016 | CT aspects of eNPN | eNPN | ct | CP-212103 | Sedlacek, Ivo, Ericsson |
| 910066 | CT1 aspects of eNPN | eNPN | C1 | CP-212103 | Sedlacek, Ivo, Ericsson |
| 910067 | CT3 aspects of eNPN | eNPN | C3 | CP-212103 | Sedlacek, Ivo, Ericsson |
| 910068 | CT4 aspects of eNPN | eNPN | C4 | CP-212103 | Sedlacek, Ivo, Ericsson |
| 920025 | Security Aspects of eNPN | eNPN | S3 | SP-210422 | Jost, Christine, Ericsson |
| 880008 | Study on enhanced security support for Non-Public Networks | FS_eNPN_SEC | S3 | SP-200353 | Normann, Henrik, Ericsson |
| 890049 | Enhancement of Private Network support for NG-RAN | NG_RAN_PRN_enh | R3 | RP-212585 | China Telecom |
| 890149 | Core part: Enhancement of Private Network support for NG-RAN | NG_RAN_PRN_enh-Core | R3 | RP-212585 | China Telecom |

Summary based on the input provided by XXX in xP-21xxxx.

## 5.5	Industrial IoT

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 850012 | Study on enhanced support of Industrial IoT | FS_IIoT | S2 | SP-200298 | Nokia, Devaki Chandramouli, |
| 880010 | Study on security for enhanced support of Industrial IoT | FS_IIoT_SEC | S3 | SP-200355 | Nokia, Anja Jerichow |
| 920024 | Security for enhanced support of Industrial IoT | IIoT_SEC | S3 | SP-210421 | Nokia, Anja Jerichow |
| 910059 | Support of Enhanced Industrial IIoT | IIoT |  | SP-200973 | Devaki Chandramouli, Nokia |
| 900008 | Stage 2 for IIoT | IIoT | S2 | SP-200973 | Devaki Chandramouli, Nokia |
| 910014 | CT aspects of support of enhanced IIoT | IIoT | ct | CP-212100 | Won, Sung Hwan, Nokia |
| 910060 | CT1 aspects of support of enhanced IIoT | IIoT | C1 | CP-212100 | Won, Sung Hwan, Nokia |
| 910061 | CT3 aspects of support of enhanced IIoT | IIoT | C3 | CP-212100 | Won, Sung Hwan, Nokia |
| 910062 | CT4 aspects of support of enhanced IIoT | IIoT | C4 | CP-212100 | Won, Sung Hwan, Nokia |
| 860044 | Additional enhancements for NB-IoT and LTE-MTC | NB_IOTenh4_LTE_eMTC6 |  | RP-211340 | Huawei |
| 860144 | Core part: Additional enhancements for NB-IoT and LTE-MTC | NB_IOTenh4_LTE_eMTC6-Core | R1 | RP-211340 | Huawei |
| 860244 | Perf. part: Additional enhancements for NB-IoT and LTE-MTC | NB_IOTenh4_LTE_eMTC6-Perf | R4 | RP-211340 | Huawei |
| 860051 | NR small data transmissions in INACTIVE state | NR_SmallData_INACTIVE |  | RP-212594 | ZTE |
| 860151 | Core part: NR small data transmissions in INACTIVE state | NR_SmallData_INACTIVE-Core | R2 | RP-212594 | ZTE |
| 860251 | Perf. part: NR small data transmissions in INACTIVE state | NR_SmallData_INACTIVE-Perf | R4 | RP-212594 | ZTE |
| 860045 | Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR | NR_IIOT_URLLC_enh |  | RP-210854 | Nokia |
| 860145 | Core part: Enhanced IoT and URLLC support for NR | NR_IIOT_URLLC_enh-Core | R2 | RP-210854 | Nokia |
| 860245 | Perf. Part: Enhanced IoT and URLLC support for NR | NR_IIOT_URLLC_enh-Perf | R4 | RP-210854 | Nokia |

Summary based on the input provided by XXX in xP-21xxxx.

## 5.6	Media production

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 840045 | Audio-Visual Service Production | AVPROD |  | SP-190304 | Wagdin, Ian, BBC |
| 800014 | Study on AVPROD | FS_AVPROD | S1 | SP-181015 | Roland Beutler, EBU |
| 840031 | Stage 1 of AVPROD | AVPROD | S1 | SP-191041 | Wagdin, Ian, BBC |
| 910001 | Study on Media Production over 5G NPN | FS_NPN4AVProd | S4 | SP-210241 | Lohmar, Thorsten, Ericsson |

Summary based on the input provided by XXX in xP-21xxxx.

See also "New bands and bandwidth allocation for 5G terrestrial broadcast - part 1"

## 5.7	Other services to verticals

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 840050 | Enhancements for cyber-physical control applications in vertical domains | eCAV |  | SP-190310 | Bahr, Michael, Siemens AG |
| 830020 | Study on eCAV | FS_eCAV | S1 | SP-190092 | Michael Bahr, Siemens AG |
| 840041 | Stage 1 of eCAV | eCAV | S1 | SP-191043 | Bahr, Michael, Siemens AG |

Summary based on the input provided by XXX in xP-21xxxx.

| 850046 | Asset Tracking for 5G | ATRAC |  | SP-190816 | Thierry Berisot, NOVAMINT |
| --- | --- | --- | --- | --- | --- |
| 810017 | Study on ATRAC | FS_5G_ATRAC | S1 | SP-180922 | Thierry Berisot, NOVAMINT |
| 850037 | Stage 1 of ATRAC | ATRAC | S1 | SP-190931 | Thierry Berisot, NOVAMINT |

| 840047 | Communication Service Requirements for Critical Medical Applications | CMED |  | SP-190306 | Lagrange, Mathieu, b<>com |
| --- | --- | --- | --- | --- | --- |
| 810016 | Study on CMED | FS_CMED | S1 | SP-180783 | Mathieu Lagrange, b<>com |
| 840033 | Stage 1 of CMED | CMED | S1 | SP-190306 | Lagrange, Mathieu, b<>com |

Summary based on the input provided by XXX in xP-21xxxx.

| 820025 | Study on application layer support for Factories of the Future in 5G network | FS_FFAPP | S6 | SP-200836 | Shao Weixiang, ZTE Corporation |
| --- | --- | --- | --- | --- | --- |
| 840025 | Study on enhancement of support for 5G LAN-type service | FS_5GLAN_enh | S2 | SP-190626 | Runze Zhou, Huawei |
| 900022 | Study on Charging Aspect of 5G LAN-type Services | FS_5GLAN_CH | S5 | SP-201081 | CHEN SHAN, Huawei |

# 6	5G access via Satellite/Non-Terrestrial (NTN) link

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 890034 | Integration of satellite components in the 5G architecture | 5GSAT_ARCH |  | SP-191335 | Cyril MICHEL, thales |
| 800048 | Stage 1 of 5GSAT | 5GSAT | S1 | SP-180326 | MICHEL, Cyril, Thales |
| 800026 | Study on architecture aspects for using satellite access in 5G | FS_5GSAT_ARCH | S2 | SP-181253 | MICHEL, Cyril |
| 860005 | (Stage 2 of) Integration of satellite components in the 5G architecture | 5GSAT_ARCH | S2 | SP-191335 | Cyril MICHEL, thales |
| 911030 | CT aspects of 5GC architecture for satellite networks | 5GSAT_ARCH-CT | ct | CP-210149 | Catovic, Amer, Qualcomm |
| 890005 | CT1 aspects of 5GC architecture for satellite networks | 5GSAT_ARCH-CT | C1 | CP-210149 | Catovic, Amer, Qualcomm |
| 920057 | CT3 aspects of 5GC architecture for satellite networks | 5GSAT_ARCH-CT | C3 | CP-210149 | Catovic, Amer, Qualcomm |
| 911031 | CT4 aspects of 5GC architecture for satellite networks | 5GSAT_ARCH-CT | C4 | CP-210149 | Catovic, Amer, Qualcomm |
| 930044 | CT6 aspects of 5GC architecture for satellite networks | 5GSAT_ARCH-CT | C6 | CP-210149 | Catovic, Amer, Qualcomm |
| 860046 | Solutions for NR to support non-terrestrial networks (NTN) | NR_NTN_solutions |  | RP-210908 | Thales |
| 860146 | Core part: Solutions for NR for NTN | NR_NTN_solutions-Core | R2 | RP-210908 | Thales |
| 860246 | Perf. Part: Solutions for NR for NTN | NR_NTN_solutions-Perf | R4 | RP-210908 | Thales |
| 920069 | NB-IoT/eMTC support for Non-Terrestrial Networks | LTE_NBIOT_eMTC_NTN | R1 | RP-211601 | MediaTek |
| 920169 | Core part: NB-IoT/eMTC support for Non-Terrestrial Networks | LTE_NBIOT_eMTC_NTN-Core | R1 | RP-211601 | MediaTek |
| 930019 | Architecture support for NB-IoT/eMTC Non-Terrestrial Networks in EPS | IoT_SAT_ARCH_EPS | S2 | SP-211124 | SEBIRE, Guillaume, MediaTek Inc. |
| 830025 | Study on management and orchestration aspects with integrated satellite components in a 5G network | FS_5GSAT_MO | S5 | SP-190138 | Floris Drijver, TNO |
| 860033 | Study on NB-IoT/eMTC support for NTN | FS_LTE_NBIOT_eMTC_NTN | R1 | RP-202689 | MediaTek |

Summary based on the input provided by XXX in xP-21xxxx.

# 7	V2V/D2D/Sidelink related

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 860042 | NR Sidelink enhancement | NR_SL_enh |  | RP-202846 | LG Electronics |
| 860142 | Core part: NR Sidelink enhancement | NR_SL_enh-Core | R1 | RP-202846 | LG Electronics |
| 860242 | Perf. part: NR Sidelink enhancement | NR_SL_enh-Perf | R4 | RP-202846 | LG Electronics |
| 860038 | Study on NR Sidelink relay | FS_NR_SL_relay | R2 | RP-202208 | OPPO |
| 840035 | Study on enhancements to application layer support for V2X services | FS_eV2XAPP | S6 | SP-200110 | Niranth Amogh, Huawei Telecommunications India |
| 910075 | Enhanced application layer support for V2X services | eV2XAPP |  | SP-200831 | Niranth Amogh, Huawei Telecommunications India |
| 890036 | Stage 2 of eV2XAPP | eV2XAPP | S6 | SP-200831 | Niranth Amogh, Huawei Telecommunications India |
| 910019 | CT aspects of eV2XAPP | eV2XAPP | ct | CP-211109 | Herrero Veron, Christian (Huawei) |
| 910076 | CT1 aspects of eV2XAPP | eV2XAPP | C1 | CP-211109 | Herrero Veron, Christian (Huawei) |
| 910077 | CT3 aspects of eV2XAPP | eV2XAPP | C3 | CP-211109 | Herrero Veron, Christian (Huawei) |
| 910037 | Support of advanced V2X services - Phase 2 | eV2XARC_Ph2 |  | SP-210090 | LaeYoung Kim, LG Electronics |
| 850013 | Study on V2X services – Phase 2 | FS_eV2XARC_Ph2 | S2 | SP-190631 | LaeYoung Kim, LG Electronics |
| 910021 | Stage 2 of eV2XARC_Ph2 | eV2XARC_Ph2 | S2 | SP-210090 | LaeYoung Kim, LG Electronics |
| 920005 | CT1 aspects of eV2XARC_Ph2 | eV2XARC_Ph2 | C1 | CP-211116 | Herrero Veron, Christian (Huawei) |
| 920047 | CT3 aspects of eV2XARC_Ph2 | eV2XARC_Ph2 | C3 | CP-211116 | Herrero Veron, Christian (Huawei) |
| 920048 | CT6 aspects of eV2XARC_Ph2 | eV2XARC_Ph2 | C6 | CP-211116 | Herrero Veron, Christian (Huawei) |
| 900030 | Proximity based Services in 5GS | 5G_ProSe |  | SP-200972 | TBD |
| 830033 | Study on System enhancement for Proximity based Services in 5GS | FS_5G_ProSe | S2 | SP-190443 | Qiang Deng, CATT |
| 900007 | Stage 2 for Proximity based Services in 5GS | 5G_ProSe | S2 | SP-201096 | Qiang Deng, CATT |
| 910018 | CT aspects of proximity based services in 5GS | 5G_ProSe | ct | CP-212105 | Yong Jiang, CATT |
| 910072 | CT1 aspects of 5G_ProSe | 5G_ProSe | C1 | CP-212105 | Yong Jiang, CATT |
| 910073 | CT3 aspects of 5G_ProSe | 5G_ProSe | C3 | CP-212105 | Yong Jiang, CATT |
| 910074 | CT4 aspects of 5G_ProSe | 5G_ProSe | C4 | CP-212105 | Yong Jiang, CATT |
| 920081 | CT6 aspects of 5G_ProSe | 5G_ProSe | C6 | CP-212105 | Yong Jiang, CATT |
| 890018 | Study on charging aspects of Proximity-based Services in 5GS | FS_5G_Prose_CH | S5 | SP-200767 | Shu, Min, CATT |
| 880005 | Study on Security Aspects of Enhancement for Proximity Based Services in 5GS | FS_5G_ProSe_Sec | S3 | SP-200350 | Wei Zhou, CATT |
| 930008 | Security Aspects of Proximity based Services in 5GS | 5G_ProSe | S3 | SP-211120 | Wei Zhou, CATT |
| 911005 | NR Sidelink Relay | NR_SL_relay |  | RP-212601 | OPPO |
| 911105 | Core part: NR Sidelink Relay | NR_SL_relay-Core | R2 | RP-211050 | OPPO |
| 911205 | Perf. part: NR Sidelink Relay | NR_SL_relay-Perf | R4 | RP-211050 | OPPO |

Summary based on the input provided by XXX in xP-21xxxx.

# 8	Enhancements to Network architecture and application enablement

## 8.1	Edge computing

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 880042 | Architecture for enabling Edge Applications | EDGEAPP |  | SP-200109 | Gupta, Nishant, Samsung |
| 830008 | Study on Application Architecture for enabling Edge Applications | FS_EDGEAPP | S6 | SP-190065 | Nishant Gupta, Samsung |
| 860006 | Architecture for enabling Edge Applications | EDGEAPP | S6 | SP-200886 | Gupta, Nishant, Samsung |
| 900006 | CT aspects for Enabling Edge Applications | EDGEAPP |  | CP-211196 | Narendranath Durga Tangudu |
| 900034 | CT1 aspects for Enabling Edge Applications | EDGEAPP | C1 | CP-211196 | Narendranath Durga Tangudu |
| 900035 | CT3 aspects for Enabling Edge Applications | EDGEAPP | C3 | CP-211196 | Narendranath Durga Tangudu |
| 830032 | Study on enhancement of support for Edge Computing in 5GC | FS_enh_EC | S2 | SP-200093 | Hui Ni, Huawei |
| 910048 | Enhancement of support for Edge Computing in 5G Core network | eEDGE_5GC |  | SP-201107 | Hui Ni, Huawei Technologies |
| 900016 | Stage 2 of eEDGE_5GC | eEDGE_5GC | S2 | SP-201107 | Hui Ni, Huawei Technologies |
| 910005 | CT aspects of 5G eEDGE | eEDGE_5GC | ct | CP-212021 | Qi Caixia, Huawei |
| 910049 | CT1 aspects of 5G eEDGE | eEDGE_5GC | C1 | CP-212021 | Qi Caixia, Huawei |
| 910050 | CT3 aspects of 5G eEDGE | eEDGE_5GC | C3 | CP-212021 | Qi Caixia, Huawei |
| 910051 | CT4 aspects of 5G eEDGE | eEDGE_5GC | C4 | CP-212021 | Qi Caixia, Huawei |
| 920026 | Security Aspects of Enhancements of Support for Edge Computing in 5GC | eEDGE_5GC | S3 | SP-210423 | Bo Zhang, Huawei Technologies |
| 920019 | Edge Computing Management | ECM | S5 | SP-210388 | Deepanshu Gautam, Samsung |
| 880002 | Study on Security Aspects of Enhancement of Support for Edge Computing in 5GC | FS_eEDGE_Sec | S3 | SP-200347 | Bo Zhang, Huawei Technologies |
| 930034 | Charging aspects of Edge Computing | EDGE_CH | S5 | SP-210861 | Yao, Yizhi, Intel |
| 880030 | Study on charging aspects of Edge Computing | FS_EDGE_CH | S5 | SP-200467 | Yizhi Yao, Intel |
| 930011 | Charging aspects of Edge Computing | EDGE_CH | S5 | SP-210861 | Yao, Yizhi, Intel |
| 870029 | Study on enhancements of edge computing management | FS_eEDGE_Mgt | S5 | SP-200195 | Joey Chou, Intel |

Summary based on the input provided by XXX in xP-21xxxx.

## 8.2	Slicing

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 900032 | Enhancement of Network Slicing Phase 2 | eNS_Ph2 |  | SP-200976 | ZTE, Jinguo Zhu |
| 850010 | Study on Enhancement of Network Slicing Phase 2 | FS_eNS_Ph2 | S2 | SP-190931 | So, Tricci, ZTE |
| 900011 | Stage 2 for Enhancement of Network Slicing Phase 2 | eNS_Ph2 | S2 | SP-200976 | ZTE, Jinguo Zhu |
| 920059 | Stage 3 for Enhancement of Network Slicing Phase 2 | eNS_Ph2 | ct | CP-211091 | Hannah Wang, ZTE |
| 910041 | CT1 aspects of eNS_Ph2 | eNS_Ph2 | C1 | CP-211091 | Hannah Wang, ZTE |
| 910042 | CT3 aspects of eNS_Ph2 | eNS_Ph2 | C3 | CP-211091 | Hannah Wang, ZTE |
| 910043 | CT4 aspects of eNS_Ph2 | eNS_Ph2 | C4 | CP-211091 | Hannah Wang, ZTE |
| 911007 | Enhancement of RAN slicing for NR | NR_slice | R2 | RP-212534 | CMCC |
| 911107 | Core part: Enhancement of RAN slicing for NR | NR_slice-Core | R2 | RP-212534 | CMCC |
| 860022 | Study on network slice management enhancement | FS_NSMEN | S5 | SP-200766 | Brendan Hassett, Huawei Technologies Sweden AB, |
| 900023 | Study on Charging Aspects for Network Slicing Phase 2 | FS_NETSLICE_CH_Ph2 | S5 | SP-201082 | Matrixx, Gerald Görmer |
| 910023 | Study on enhanced security for network slicing Phase 2 | FS_eNS2_SEC | S3 | SP-210106 | Zander Lei, Huawei |
| 910026 | Study on network slice management capability exposure | FS_NSCE | S5 | SP-210131 | Xiaobo Yu, Alibaba Group |
| 910095 | Study on 5G NR UE full stack testing for Network Slicing | FS_NR_Slice_Test | R5 | RP-211977 | CMCC |

Summary based on the input provided by XXX in xP-21xxxx.

## 8.3	Access Traffic Steering, Switch and Splitting support in the 5G system architecture; Phase 2

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 900033 | Access Traffic Steering, Switch and Splitting support in the 5G system architecture; Phase 2 | ATSSS_Ph2 |  | SP-200977 | Apostolis Salkintzis, Lenovo |
| 840084 | Study on ATSSS_Ph2 | FS_ATSSS_Ph2 | S2 | SP-200095 | So, Tricci, ZTE |
| 900012 | Stage 2 of ATSSS_Ph2 | ATSSS_Ph2 | S2 | SP-200977 | Apostolis Salkintzis, Lenovo |
| 910013 | CT aspects of ATSSS_Ph2 | ATSSS_Ph2 | ct | CP-210136 | ZHOU Xingyue (Joy), ZTE |
| 910056 | CT1 aspects of ATSSS_Ph2 | ATSSS_Ph2 | C1 | CP-210136 | ZHOU Xingyue (Joy), ZTE |
| 910057 | CT3 aspects of ATSSS_Ph2 | ATSSS_Ph2 | C3 | CP-210136 | ZHOU Xingyue (Joy), ZTE |
| 910058 | CT4 aspects of ATSSS_Ph2 | ATSSS_Ph2 | C4 | CP-210136 | ZHOU Xingyue (Joy), ZTE |

Summary based on the input provided by XXX in xP-21xxxx.

## 8.4	Self-Organizing (SON)/Autonomous Network

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 860053 | Enhancement of data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR and EN-DC | NR_ENDC_SON_MDT_enh |  | RP-193255 | CMCC |
| 860153 | Core part: Enhancement of data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR and EN-DC | NR_ENDC_SON_MDT_enh-Core | R3 | RP-193255 | CMCC |
| 910029 | Plug and connect support for management of Network Functions | PACMAN | S5 | SP-210260 | Per Elmdahl, Ericsson |
| 870028 | Enhancements of Self-Organizing Networks (SON) for 5G networks | eSON_5G | S5 | SP-200194 | Joey Chou, Intel |
| 850032 | Study on autonomous network levels | FS_ANL | S5 | SP-190928 | China Mobile |
| 880027 | Autonomous network levels | ANL | S5 | SP-200464 | Cao, Xi, China Mobile |
| 900031 | Enablers for Network Automation for 5G - phase 2 | eNA_Ph2 |  | SP-200975 | TBD |
| 840022 | Study on Enablers for Network Automation for 5G - phase 2 | FS_eNA_Ph2 | S2 | SP-200098 | Xiaobo Wu, Huawei Technologies |
| 890015 | Study on security aspects of enablers for Network Automation (eNA) for 5GS Phase 2 | FS_eNA_SEC | S3 | SP-200722 | Xiaoting Huang, China Mobile |
| 900010 | Stage 2 of eNA_Ph2 | eNA_Ph2 | S2 | SP-200975 | TBD |
| 910012 | CT aspects of eNA_Ph2 | eNA_Ph2 | ct | CP-211191 | Huang Zhenning (China Mobile) |
| 910088 | CT3 aspects of eNA_Ph2 | eNA_Ph2 | C3 | CP-211191 | Huang Zhenning (China Mobile) |
| 910089 | CT4 aspects of eNA_Ph2 | eNA_Ph2 | C4 | CP-211191 | Huang Zhenning (China Mobile) |
| 930007 | Security aspects of eNA_Ph2 | eNA_Ph2 | S3 | SP-210837 | Chang Liu, China Mobile |

Summary based on the input provided by XXX in xP-21xxxx.

## 8.5	Minimization of service Interruption

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 850045 | Support for Minimization of service Interruption | MINT |  | SP-190814 | SungDuck Chun; LG Electronics |
| 830018 | Study on MINT | FS_MINT | S1 | SP-190090 | SungDuck Chun; LG Electronics |
| 850036 | Stage 1 of MINT | MINT | S1 | SP-190938 | SungDuck Chun; LG Electronics |
| 920062 | Stage 2 of MINT | MINT | S2 | SP-210582 | Hyunsook Kim, LG Electronics |
| 900004 | Study on the CT aspects of Support for Minimization of service Interruption | FS_MINT-CT | C1 | CP-203232 | Sang Min Park, LG Electronics |
| 930003 | Stage 3 of MINT | MINT | ct | CP-212166 | Sang Min Park |
| 930049 | CT1 aspects of MINT | MINT | C1 | CP-212166 | Sang Min Park |
| 930045 | CT4 aspects of MINT | MINT | C4 | CP-212166 | Sang Min Park |
| 930048 | CT6 aspects of MINT | MINT | C6 | CP-212166 | Sang Min Park |

Summary based on the input provided by XXX in xP-21xxxx.

# 9	Multicast-Broadcast Services (MBS) and Mission Critical (MC)

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 900038 | Multicast-broadcast services in 5G | 5MBS |  | SP-201106 | Meng Li, Huawei |
| 830030 | Study on Architectural enhancements for 5G multicast-broadcast services | FS_5MBS | S2 | SP-200690 | Meng Li, Huawei |
| 900009 | Stage 2 for 5MBS | 5MBS | S2 | SP-201106 | Meng Li, Huawei |
| 920043 | CT1 aspects of 5MBS | 5MBS | C1 | CP-212022 | Gulbani, Giorgi, Huawei |
| 920044 | CT3 aspects of 5MBS | 5MBS | C3 | CP-212022 | Gulbani, Giorgi, Huawei |
| 910002 | CT4 aspects of 5MBS | 5MBS | C4 | CP-212022 | Gulbani, Giorgi, Huawei |
| 920023 | Security Aspects of Enhancements for 5G Multicast-Broadcast Services | 5MBS | S3 | SP-210420 | Longhua Guo, Huawei Technologies |
| 870014 | Study on Multicast Architecture Enhancements for 5G Media Streaming | FS_5GMS_Multicast | S4 | SP-200238 | Peng Tan, Telus |
| 810055 | Study on location enhancements for mission critical services | FS_enhMCLoc | S6 | SP-190725 | Dom Lazara, Motorola Solutions |
| 850040 | Broadcast / Multicast requirements supporting Mission Critical Services in 5G | 5MBS_eMC | S1 | SP-190942 | Toobe, Jens, BDBOS |
| 880006 | Study on Security Aspects of Enhancements for 5G Multicast-Broadcast Services | FS_5MBS_SEC | S3 | SP-200351 | Longhua Guo, Huawei Technologies |
| 850035 | Study on Mission Critical services over 5G multicast-broadcast system | FS_MC5MBS | S6 | SP-190929 | Val Oprescu, AT&T |
| 860048 | NR multicast and broadcast services | NR_MBS | R2 | RP-193248 | Huawei |
| 860148 | Core part: NR multicast and broadcast services | NR_MBS-Core | R2 | RP-201038 | Huawei |
| 800023 | Study on Mission Critical services support over 5G System | FS_MCOver5GS | S6 | SP-200837 | Verweij, Kees, The Police of the Netherlands |
| 920051 | Mission Critical Services over 5GS | MCOver5GS |  | SP-200833 | Wendler, Ingo, UIC |
| 890027 | Stage 2 of MCOver5GS | MCOver5GS | S6 | SP-200833 | Wendler, Ingo, UIC |
| 920007 | CT1 aspects of MCOver5GS | MCOver5GS | C1 | CP-211118 | Gkatzikis, Lazaros, Nokia |
| 890039 | Mission Critical Data | eMCData3 |  | SP-191106 | Shih, Jerry, AT&T |
| 860007 | Stage 2 of MCData3 | eMCData3 | S6 | SP-191106 | Shih, Jerry, AT&T |
| 890038 | CT aspects of eMCData3 | eMCData3 | C1 | CP-201177 |  |
| 880020 | Protocol enhancements for Mission Critical Services | MCProtoc17 | C1 | CP-202193 | AXELL, Jörgen |
| 820040 | Mission Critical system migration and interconnection | MCSMI_CT | C1 | CP-190143 | Dom Lazara, Motorola Solutions |
| 890003 | CT aspects of Enhanced Mission Critical Communication Interworking with Land Mobile Radio Systems | eMCCI_CT | C1 | eMCCI_CT | Mike Dolan, FirstNet |

Summary based on the input provided by XXX in xP-21xxxx.

# 10	Multi-(U)SIM

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 840049 | Support for Multi-USIM Devices | MUSIM |  | SP-190309 | Liao, Ellen C, Intel |
| 830019 | Study on MUSIM | FS_MUSIM | S1 | SP-190091 | Liao, Ellen C, Intel |
| 840040 | Stage 1 of MUSIM | MUSIM | S1 | SP-190309 | Liao, Ellen C, Intel |
| 820012 | Study on Stage 2 (System Enablers) for MUSIM | FS_MUSIM | S2 | SP-200297 | Sašo Stojanovski, Intel |
| 900013 | System enablers for Multi-USIM devices | MUSIM | S2 | SP-210091 | Sašo Stojanovski, Intel |
| 910015 | CT aspects of MUSIM | MUSIM | ct | CP-212102 | Vivek Gupta, Intel |
| 910063 | CT1 aspects of Enabling Multi-USIM Devices | MUSIM | C1 | CP-212102 | Vivek Gupta, Intel |
| 910064 | CT4 aspects of Enabling Multi-USIM Devices | MUSIM | C4 | CP-212102 | Vivek Gupta, Intel |
| 900017 | Study on the security of the system enablers for devices having multiple Universal Subscriber Identity Modules (USIM) | FS_MUSIM_SEC | S3 | SP-201018 | Abhijeet Kolekar, Intel Corporation |
| 860063 | Support for Multi-SIM devices for LTE/NR | LTE_NR_MUSIM |  | RP-212610 | Vivo |
| 860163 | Core part: Support for Multi-SIM devices for LTE/NR | LTE_NR_MUSIM-Core | R2 | RP-212610 | Vivo |

Summary based on the input provided by XXX in xP-21xxxx.

# 11	MSG in 5G

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 930030 | Support of the 5GMSG Service | FS_5GMARCH | S6 | SP-200835 | Liu, Yue, China Mobile |
| 840036 | Study on support of the 5GMSG Service | FS_5GMARCH | S6 | SP-200835 | Liu, Yue, China Mobile |
| 890010 | Study on security aspects of the 5GMSG Service (St1 in Rel16) | FS_SEC_5GMSG | S3 | SP-200878 | Xiaoting Huang, China Mobile |
| 930005 | Security aspects of the 5GMSG Service | 5GMSG | S3 | SP-210835 | Xiaoting Huang, China Mobile |
| 930040 | Application Architecture for MSGin5G Service | 5GMARCH |  | SP-200832 | Liu, Yue, China Mobile |
| 890026 | Application Architecture for MSGin5G Service | 5GMARCH | S6 | SP-200832 | Liu, Yue, China Mobile |
| 930004 | CT aspects for enabling MSGin5G Service | 5GMARCH | ct | CP-212106 | Liu, Yue, China Mobile |
| 930041 | CT1 aspects for enabling MSGin5G Service | 5GMARCH | C1 | CP-212106 | Liu, Yue, China Mobile |
| 930042 | CT3 aspects for enabling MSGin5G Service | 5GMARCH | C3 | CP-212106 | Liu, Yue, China Mobile |

Summary based on the input provided by XXX in xP-21xxxx.

# 12	Drone/UAS/UAV/EAV identification

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 840083 | 5G Enhancement for UAVs | EAV |  | SP-190308 | Qun Wei, China Unicom |
| 810019 | Study on EAV | FS_EAV | S1 | SP-180909 | Qun Wei, China Unicom |
| 840039 | Stage 1 of EAV | EAV | S1 | SP-190308 | Qun Wei, China Unicom |
| 820011 | Study on supporting Uncrewed Aerial Systems Connectivity, Identification, and Tracking | FS_ID_UAS_SA2 | S2 | SP-200097 | Qualcomm, Stefano Faccin |
| 820026 | Study on application layer support for Uncrewed Aerial System (UAS) | FS_UASAPP | S6 | SP-200111 | Niranth Amogh, Huawei |
| 920045 | Application layer support for Uncrewed Aerial System (UAS) | UASAPP |  | SP-200988 | Niranth Amogh, Huawei Telecommunications India |
| 900025 | Stage 2 of Application layer support for Uncrewed Aerial System (UAS) | UASAPP | S6 | SP-200988 | Niranth Amogh, Huawei Telecommunications India |
| 920004 | CT1 Aspects of Application Layer Support for Uncrewed Aerial Systems (UAS) | UASAPP | C1 | CP-211111 | Lin Shu, Huawei, |
| 920046 | CT3 Aspects of Application Layer Support for Uncrewed Aerial Systems (UAS) | UASAPP | C3 | CP-211111 | Lin Shu, Huawei |
| 880007 | Study on security aspects of Uncrewed Aerial Systems (UAS) | FS_UAS_SEC | S3 | SP-200352 | Adrian Escott, Qualcomm |
| 810049 | Remote Identification of Uncrewed Aerial Systems | ID_UAS |  | SP-180771 | Eddy HALL, Qualcomm |
| 810013 | Stage 1 of Remote Identification of Uncrewed Aerial Systems | ID_UAS | S1 | SP-180771 | Eddy HALL, Qualcomm |
| 900014 | (Stage 2 of) Support of Uncrewed Aerial Systems Connectivity, Identification, and Tracking | ID_UAS | S2 | SP-200979 | Faccin, Stefano, Qualcomm |
| 910017 | CT aspects for ID_UAS | ID_UAS | ct | CP-211112 | Sunghoon Kim, Qualcomm |
| 910069 | CT1 aspects for ID_UAS | ID_UAS | C1 | CP-211112 | Sunghoon Kim, Qualcomm |
| 910070 | CT3 aspects for ID_UAS | ID_UAS | C3 | CP-211112 | Sunghoon Kim, Qualcomm |
| 910071 | CT4 aspects for ID_UAS | ID_UAS | C4 | CP-211112 | Sunghoon Kim, Qualcomm |
| 920028 | Security aspects of Uncrewed Aerial Systems | ID_UAS | S3 | SP-210425 | Escott, Adrian, Qualcomm |

Summary based on the input provided by XXX in xP-21xxxx.

# 13	Authentication and key management for applications based on 3GPP credential in 5G

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 890030 | Authentication and key management for applications based on 3GPP credential in 5G | AKMA |  | SP-190711 | Xiaoting Huang, China Mobile |
| 850021 | SA3 aspects of AKMA | AKMA | S3 | SP-190711 | Xiaoting Huang, China Mobile |
| 890008 | CT aspects of AKMA | AKMA-CT | ct | CP-203107 | Huang Zhenning (China Mobile) |
| 890031 | CT1 aspects of AKMA | AKMA-CT | C1 | CP-203107 | Huang Zhenning (China Mobile) |
| 890032 | CT3 aspects of AKMA | AKMA-CT | C3 | CP-203107 | Huang Zhenning (China Mobile) |
| 890033 | CT4 aspects of AKMA | AKMA-CT | C4 | CP-203107 | Huang Zhenning (China Mobile) |

Summary based on the input provided by XXX in xP-21xxxx.

# 14	Multimedia Priority Service (MPS) Phase 2

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 840046 | Multimedia Priority Service (MPS) Phase 2 | MPS2 |  | SP-190305 | Singh, Ray P; Perspecta Labs |
| 780005 | Study on MPS2 | FS_MPS2 | S1 | SP-190315 | Singh, Ray P; Vencore Labs |
| 850011 | Study on MPS2, Stage 2 | FS_MPS2_St2 | S2 | SP-190629 | Don Lukacs; Perspecta Labs |
| 840032 | Stage 1 of MPS2 | MPS2 | S1 | SP-190561 | Singh, Ray P; Perspecta Labs |
| 870002 | Stage 2 of MPS2 | MPS2 | S2 | SP-200519 | Streijl, Robert, Perspecta Labs |
| 880031 | Stage 3 of MPS2 | MPS2 | ct | CP-201207 | Peter Monnes, Perspecta Labs |
| 880050 | CT1 aspects of MPS2 | MPS2 | C1 | CP-201207 | Peter Monnes, Perspecta Labs |
| 880058 | CT3 aspects of MPS2 | MPS2 | C3 | CP-201207 | Peter Monnes, Perspecta Labs |
| 880051 | CT4 aspects of MPS2 | MPS2 | C4 | CP-201207 | Peter Monnes, Perspecta Labs |

Summary based on the input provided by XXX in xP-21xxxx.

# 15	NR enhancements

| 890056 | Introduction of DL 1024QAM for NR frequency range 1 (FR1) | NR_DL1024QAM_FR1 |  | RP-202886 | Ericsson |
| --- | --- | --- | --- | --- | --- |
| 890156 | Core part: Introduction of DL 1024QAM for NR FR1 | NR_DL1024QAM_FR1-Core | R4 | RP-202886 | Ericsson |
| 890256 | Perf. part: Introduction of DL 1024QAM for NR FR1 | NR_DL1024QAM_FR1-Perf | R4 | RP-202886 | Ericsson |

Summary based on the input provided by Ericsson in RP-213003.

This work item [1] has specified downlink 1024QAM for NR PDSCH operation in FR1, which provides higher downlink peak rate compared with Release-15 NR, with the high downlink SINR and better channel condition (e.g., LOS or LOS-like channel), and with no mobility or very low mobility environment.

This WI specified the downlink 1024QAM mapping function, and the corresponding MCS/CQI tables and RRC signalling. This WI also specified the corresponding BS/UE RF requirements to transmit/receive the signals with 1024QAM [2].

Introduction of the modulation mapping function of 1024QAM

It was introduced the modulation mapping function of 1024QAM for PDSCH, where 10 tuples of bits, bi, …, bi+9 are mapped to complex-valued modulation symbols d(i), according to

di=16821-2bi16-1-2bi+28-1-2bi+44-1-2bi+62-1-2bi+8+j16821-2bi+116-1-2bi+38-1-2bi+54-1-2bi+72-1-2bi+9

Introduction of MCS table supporting 1024QAM

For supporting 1024QAM for PDSCH, a new five-bit MCS table with 1024QAM entries was introduced by removing 5 MCS entries and adding 5 new entries for 1024QAM from the existing MCS index table 2. The removed MCS indexes are 2, 4, 6, 8, and 10. The added MCS entries for 1024QAM are given as follows:

| MCS index | Modulation | Target code rate R x [1024] | Spectrum efficiency |
| --- | --- | --- | --- |
| 23 | 10 | 805.5 | 7.8662 |
| 24 | 10 | 853 | 8.3301 |
| 25 | 10 | 900.5 | 8.7939 |
| 26 | 10 | 948 | 9.2578 |
| 31 | 10 | reserved |  |

New RRC signalling was also introduced to indicate use of 1024QAM MCS table.

Introduction of CQI table supporting 1024QAM

For supporting 1024QAM for PDSCH, a new CQI table with 1024QAM entries was introduced by reusing LTE CQI table with 1024QAM entries as follows:

| CQI index | Modulation | Code rate x 1024 | Efficiency |
| --- | --- | --- | --- |
| 0 | Out of range |  |  |
| 1 | QPSK | 78 | 0.1523 |
| 2 | QPSK | 193 | 0.377 |
| 3 | QPSK | 449 | 0.877 |
| 4 | 16QAM | 378 | 1.4766 |
| 5 | 16QAM | 616 | 2.4063 |
| 6 | 64QAM | 567 | 3.3223 |
| 7 | 64QAM | 666 | 3.9023 |
| 8 | 64QAM | 772 | 4.5234 |
| 9 | 64QAM | 873 | 5.1152 |
| 10 | 256QAM | 711 | 5.5547 |
| 11 | 256QAM | 797 | 6.2266 |
| 12 | 256QAM | 885 | 6.9141 |
| 13 | 256QAM | 948 | 7.4063 |
| 14 | 1024QAM | 853 | 8.3301 |
| 15 | 1024QAM | 948 | 9.2578 |

New RRC signalling was also introduced to indicate use of 1024QAM CQI table.

BS Tx EVM for DL 1024QAM

The Error Vector Magnitude (EVM) is a measure of the difference between the ideal symbols and the measured symbols after the equalization. The required gNB Tx EVM for 1024QAM is set to 2.5% for frequencies equal to or below 4.2GHz and 2.8% for frequencies above 4.2GHz in FR1.

References

- RP-202886	“Revised WID on Introduction of DL 1024QAM for NR FR1”, Nokia.

- RP-213002	“Status report for WI Introduction of DL 1024QAM for NR FR1; rapporteur: Ericsson, Nokia”, Ericsson.

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 920070 | UE RF requirements for Transparent Tx Diversity (TxD) for NR | NR_RF_TxD | R4 | RP-211940 | Qualcomm |
| 920170 | Core part: UE RF requirements for Transparent Tx Diversity (TxD) for NR | NR_RF_TxD-Core | R4 | RP-211940 | Qualcomm |
| 920270 | Perf. part: UE RF requirements for Transparent Tx Diversity (TxD) for NR | NR_RF_TxD-Perf | R4 | RP-211940 | Qualcomm |
| 860040 | Further enhancements on MIMO for NR | NR_feMIMO |  | RP-212535 | Samsung |
| 860140 | Core part: Further enhancements on MIMO for NR | NR_feMIMO-Core | R1 | RP-212535 | Samsung |
| 860240 | Perf. part: Further enhancements on MIMO for NR | NR_feMIMO-Perf | R4 | RP-212535 | Samsung |
| 860043 | NR Dynamic spectrum sharing (DSS) | NR_DSS |  | RP-211345 | Ericsson |
| 860143 | Core part: NR DSS | NR_DSS-Core | R1 | RP-211345 | Ericsson |
| 860047 | UE power saving enhancements for NR | NR_UE_pow_sav_enh |  | RP-212630 | MediaTek |
| 860147 | Core part: UE power saving enhancements for NR | NR_UE_pow_sav_enh-Core | R2 | RP-212630 | MediaTek |
| 860247 | Perf. part: UE power saving enhancements for NR | NR_UE_pow_sav_enh-Perf | R4 | RP-212630 | MediaTek |
| 860049 | Further Multi-RAT Dual-Connectivity enhancements | LTE_NR_DC_enh2 |  | RP-201040 | Huawei |
| 860149 | Core part: Further Multi-RAT Dual-Connectivity enhancements | LTE_NR_DC_enh2-Core | R2 | RP-201040 | Huawei |
| 860249 | Perf. part: Further Multi-RAT Dual-Connectivity enhancements | LTE_NR_DC_enh2-Perf | R4 | RP-201040 | Huawei |
| 860050 | Enhancements to Integrated Access and Backhaul (IAB) for NR | NR_IAB_enh |  | RP-211548 | Qualcomm |
| 860150 | Core part: Enhancements to IAB for NR | NR_IAB_enh-Core | R2 | RP-211548 | Qualcomm |
| 860250 | Perf. part: Enhancements to IAB for NR | NR_IAB_enh-Perf | R4 | RP-211548 | Qualcomm |
| 830021 | Study on Security for NR_IAB | FS_NR_IAB_Sec | S3 | SP-201016 | Rajavelsamy Rajadurai, Samsung, |
| 860052 | Enhanced eNB(s) architecture evolution for E-UTRAN and NG-RAN | LTE_NR_arch_evo_enh |  | RP-211409 | China Unicom |
| 860152 | Core part: LTE_NR_arch_evo_enh | LTE_NR_arch_evo_enh-Core | R3 | RP-211409 | China Unicom |
| 890055 | Further enhancement on NR demodulation performance | NR_demod_enh2 |  | RP-212636 | China Telecom |
| 890255 | Perf. part: NR_demod_enh2 | NR_demod_enh2-Perf | R4 | RP-212636 | China Telecom |
| 890057 | NR RRM further enhancement | NR_RRM_enh2 |  | RP-202874 | Apple |
| 890157 | Core part: Further RRM enhancement for NR and MR-DC | NR_RRM_enh2-Core | R4 | RP-202874 | Apple |
| 890257 | Perf. part: Further RRM enhancement for NR and MR-DC | NR_RRM_enh2-Perf | R4 | RP-202874 | Apple |
| 890058 | Enhanced NR support for high speed train scenario for frequency range 1 (FR1) | NR_HST_FR1_enh |  | RP-210833 | CMCC |
| 890158 | Core part: Enhanced NR support for high speed train scenario for FR1 | NR_HST_FR1_enh-Core | R4 | RP-210833 | CMCC |
| 890258 | Perf. part: Enhanced NR support for high speed train scenario for FR1 | NR_HST_FR1_enh-Perf | R4 | RP-210833 | CMCC |
| 890059 | Further enhancements of NR RF requirements for frequency range 2 (FR2) | NR_RF_FR2_req_enh2 |  | RP-212092 | Nokia |
| 890159 | Core part: NR_RF_FR2_req_enh2 | NR_RF_FR2_req_enh2-Core | R4 | RP-212092 | Nokia |
| 890259 | Perf. part: NR_RF_FR2_req_enh2 | NR_RF_FR2_req_enh2-Perf | R4 | RP-212092 | Nokia |
| 890060 | NR support for high speed train scenario in frequency range 2 (FR2) | NR_HST_FR2 |  | RP-210800 | Samsung |
| 890160 | Core part: NR support for high speed train scenario in FR2 | NR_HST_FR2-Core | R4 | RP-210800 | Samsung |
| 890260 | Perf. Part: NR support for high speed train scenario in FR2 | NR_HST_FR2-Perf | R4 | RP-210800 | Samsung |
| 890061 | NR measurement gap enhancements | NR_MG_enh |  | RP-211591 | MediaTek |
| 890161 | Core part: NR and MR-DC measurement gap enhancements | NR_MG_enh-Core | R4 | RP-211591 | MediaTek |
| 890261 | Perf. part: NR and MR-DC measurement gap enhancements | NR_MG_enh-Perf | R4 | RP-211591 | MediaTek |
| 890062 | RF requirements enhancement for NR frequency range 1 (FR1) | NR_RF_FR1_enh |  | RP-212527 | Huawei |
| 890162 | Core part: RF requirements enhancement for NR FR1 | NR_RF_FR1_enh-Core | R4 | RP-212527 | Huawei |
| 890262 | Perf. part: RF requirements enhancement for NR FR1 | NR_RF_FR1_enh-Perf | R4 | RP-212527 | Huawei |

Summary based on the input provided by XXX in xP-21xxxx.

# 16	Other NR-only improvements

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 911008 | NR QoE magement and optimizations for diverse services | NR_QoE |  | RP-211406 | China Unicom |
| 911108 | Core part: NR QoE management and optimizations for diverse services | NR_QoE-Core | R3 | RP-211406 | China Unicom |
| 911009 | Rel-17 Power Class 2 UE for NR inter-band CA with or without SUL configurations with x (16>=x>2) bands DL and y (y=1, 2) bands UL | NR_UE_PC2_R17_CADC_SUL_xBDL_yBUL |  | RP-212182 | Huawei |
| 911109 | Core part: NR_UE_PC2_R17_CADC_SUL_xBDL_yBUL | NR_UE_PC2_R17_CADC_SUL_xBDL_yBUL-Core | R4 | RP-212182 | Huawei |
| 911209 | Perf. part: NR_UE_PC2_R17_CADC_SUL_xBDL_yBUL | NR_UE_PC2_R17_CADC_SUL_xBDL_yBUL-Perf | R4 | RP-212182 | Huawei |
| 911010 | Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC) | NR_FR1_TRP_TRS |  | RP-212030 | Vivo |
| 911110 | Core part: NR_FR1_TRP_TRS | NR_FR1_TRP_TRS-Core | R4 | RP-212030 | Vivo |
| 911210 | Perf. part: NR_FR1_TRP_TRS | NR_FR1_TRP_TRS-Perf | R4 | RP-212030 | Vivo |
| 911012 | High power UE (power class 1.5) for NR band n79 | NR_UE_PC1_5_n79 |  | RP-210843 | CMCC |
| 911112 | Core part: NR_UE_PC1_5_n79 | NR_UE_PC1_5_n79-Core | R4 | RP-210843 | CMCC |
| 911212 | Perf. part: NR_UE_PC1_5_n79 | NR_UE_PC1_5_n79-Perf | R4 | RP-210843 | CMCC |
| 930052 | UE Conformance - High power UE (power class 1.5) for NR band n79 | NR_UE_PC1_5_n79-UEConTest | R5 | RP-211979 | CMCC |
| 911013 | High power UE (power class 2) for NR band n34 | NR_UE_PC2_n34 |  | RP-210844 | CMCC |
| 911113 | Core part: NR_UE_PC2_n34 | NR_UE_PC2_n34-Core | R4 | RP-210844 | CMCC |
| 911213 | Perf. part: NR_UE_PC2_n34 | NR_UE_PC2_n34-Perf | R4 | RP-210844 | CMCC |
| 930053 | UE Conformance - High power UE (power class 2) for NR band n34 | NR_UE_PC2_n34-UEConTest | R5 | RP-211980 | CMCC |
| 911014 | High power UE (power class 2) for NR band n39 | NR_UE_PC2_n39 |  | RP-210845 | CMCC |
| 911114 | Core part: NR_UE_PC2_n39 | NR_UE_PC2_n39-Core | R4 | RP-210845 | CMCC |
| 911214 | Perf. part: NR_UE_PC2_n39 | NR_UE_PC2_n39-Perf | R4 | RP-210845 | CMCC |
| 930054 | UE Conformance - High power UE (power class 2) for NR band n39 | NR_UE_PC2_n39-UEConTest | R5 | RP-211981 | CMCC |
| 911015 | Introduction of FR2 FWA (Fixed Wireless Access) UE with maximum TRP of 23dBm for band n259 | NR_FR2_FWA_Bn259 |  | RP-210875 | SoftBank Corp., KDDI, NTT DOCOMO, Rakuten Mobile, Qualcomm |
| 911115 | Core part: NR_FR2_FWA_Bn259 | NR_FR2_FWA_Bn259-Core | R4 | RP-210875 | SoftBank Corp., KDDI, NTT DOCOMO, Rakuten Mobile, Qualcomm |
| 911215 | Perf. part: NR_FR2_FWA_Bn259 | NR_FR2_FWA_Bn259-Perf | R4 | RP-210875 | SoftBank Corp., KDDI, NTT DOCOMO, Rakuten Mobile, Qualcomm |
| 911006 | NR Uplink Data Compression (UDC) | NR_UDC |  | RP-211203 | CATT |
| 911106 | Core part: NR_UDC | NR_UDC-Core | R2 | RP-211203 | CATT |
| 910097 | Study on optimizations of pi/2 BPSK uplink power in NR | FS_NR_Opt_pi2BPSK | R4 | RP-210910 | Huawei |
| 910096 | Study on band combination handling in RAN4 | FS_NR_ENDC_combo_rules | R4 | RP-211146 | ZTE |
| 880078 | Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) requirements for NR UEs | NR_MIMO_OTA |  | RP-201800 | CAICT |
| 880178 | Core part: NR_MIMO_OTA | NR_MIMO_OTA-Core | R4 | RP-201800 | CAICT |
| 880278 | Perf. Part: NR_MIMO_OTA | NR_MIMO_OTA-Perf | R4 | RP-201800 | CAICT |
| 870064 | Introduction of FR2 FWA (Fixed Wireless Access) UE with maximum TRP (Total Radiated Power) of 23dBm for band n257 and n258 | NR_FR2_FWA_Bn257_Bn258 |  | RP-202565 | Softbank |
| 870164 | Core part: NR_FR2_FWA_Bn257_Bn258 | NR_FR2_FWA_Bn257_Bn258-Core | R4 | RP-202565 | Softbank |
| 870264 | Perf. Part: NR_FR2_FWA_Bn257_Bn258 | NR_FR2_FWA_Bn257_Bn258-Perf | R4 | RP-202565 | Softbank |
| 880092 | Adding channel bandwidth support to existing NR bands | NR_bands_R17_BWs |  | RP-212531 | Ericsson |
| 880192 | Core part: NR_bands_R17_BWs | NR_bands_R17_BWs-Core | R4 | RP-212531 | Ericsson |
| 880093 | Introduction of channel bandwidths 35MHz and 45MHz for NR FR1 | NR_FR1_35MHz_45MHz_BW |  | RP-211386 | Huawei |
| 880193 | Core part: NR_FR1_35MHz_45MHz_BW | NR_FR1_35MHz_45MHz_BW-Core | R4 | RP-211386 | Huawei |
| 880293 | Perf. Part: NR_FR1_35MHz_45MHz_BW | NR_FR1_35MHz_45MHz_BW-Perf | R4 | RP-211386 | Huawei |
| 880097 | SAR schemes for UE power class 2 (PC2) for NR inter-band Carrier Aggregation and supplemental uplink (SUL) configurations with 2 bands UL | NR_SAR_PC2_interB_SUL_2BUL |  | RP-212530 | China Telecom |
| 880197 | Core part: NR_SAR_PC2_interB_SUL_2BUL | NR_SAR_PC2_interB_SUL_2BUL-Core | R4 | RP-212530 | China Telecom |
| 880297 | Perf. Part: NR_SAR_PC2_interB_SUL_2BUL | NR_SAR_PC2_interB_SUL_2BUL-Perf | R4 | RP-212530 | China Telecom |
| 920065 | UE Conformance - SAR schemes for UE power class 2 (PC2) for NR inter-band Carrier Aggregation and supplemental uplink (SUL) configurations with 2 bands UL | NR_SAR_PC2_interB_SUL_2BUL-UEConTest | R5 | RP-211139 | China Telecom |
| 900061 | NR coverage enhancements | NR_cov_enh |  | RP-211566 | China Telecom |
| 860036 | Study on on NR coverage enhancements | FS_NR_cov_enh | R1 | RP-200861 | China Telecom |
| 900161 | Core part: NR_cov_enh | NR_cov_enh-Core | R1 | RP-211566 | China Telecom |
| 900261 | Perf. part: NR_cov_enh | NR_cov_enh-Perf | R4 | RP-211566 | China Telecom |
| 900062 | Support of reduced capability NR devices | NR_redcap |  | RP-211574 | Ericsson |
| 860035 | Study on support of reduced capability NR devices | FS_NR_redcap | R1 | RP-202704 | Ericsson |
| 900162 | Core part: NR_redcap | NR_redcap-Core | R1 | RP-211574 | Ericsson |
| 900262 | Perf. part: NR_redcap | NR_redcap-Perf | R4 | RP-211574 | Ericsson |
| 930018 | Architecture Enhancement for NR Reduced Capability Devices | ARCH_NR_REDCAP | S2 | SP-211100 | Aihua Li, China Mobile |
| 900064 | Additional NR bands for UL-MIMO power class 3 (PC3) in Rel-17 | NR_bands_UL_MIMO_PC3_R17 |  | RP-212184 | Huawei |
| 900164 | Core part: NR_bands_UL_MIMO_PC3_R17 | NR_bands_UL_MIMO_PC3_R17-Core | R4 | RP-212184 | Huawei |
| 900264 | Perf. part: NR_bands_UL_MIMO_PC3_R17 | NR_bands_UL_MIMO_PC3_R17-Perf | R4 | RP-212184 | Huawei |
| 900167 | Introduction of bandwidth combination set 4 (BCS4) for NR | NR_BCS4-Core | R4 | RP-202832 | Ericsson |
| 900068 | Downlink interruption for NR and EN-DC band combinations to conduct dynamic Tx Switching in Uplink | DL_intrpt_combos_TxSW_R17 |  | RP-210478 | China Telecom |
| 900168 | Core part: DL_intrpt_combos_TxSW_R17 | DL_intrpt_combos_TxSW_R17-Core | R4 | RP-202885RP-210478 | China Telecom |
| 900268 | Perf. part: DL_intrpt_combos_TxSW_R17 | DL_intrpt_combos_TxSW_R17-Perf | R4 | RP-202885RP-210478 | China Telecom |
| 900069 | High-power UE (power class 1.5) operation in NR bands n77 and n78 | HPUE_PC1_5_n77_n78 |  | RP-202912 | Qualcomm |
| 900169 | Core part: HPUE_PC1_5_n77_n78 | HPUE_PC1_5_n77_n78-Core | R4 | RP-202912 | Qualcomm |
| 930055 | UE Conformance - High-power UE (power class 1.5) operation in NR bands n77 and n78 | HPUE_PC1_5_n77_n78-UEConTest | R5 | RP-212581 | Verizon |
| 900070 | NR repeaters | NR_repeaters |  | RP-212129 | Qualcomm |
| 900170 | Core part: HPUE_PC1_5_n77_n78 | NR_repeaters-Core | R4 | RP-212129 | Qualcomm |
| 930250 | Perf. part: NR repeaters | NR_repeaters-Perf | R4 | RP-212129 | Qualcomm |
| 930056 | Increasing UE power high limit for CA and DC | Power_Limit_CA_DC | R4 | RP-212622 | Qualcomm |
| 930156 | Core part: Increasing UE power high limit for CA and DC | Power_Limit_CA_DC-Core | R4 | RP-212622 | Qualcomm |
| 930057 | Introduction of upper 700MHz A block E-UTRA band for the US | LTE_upper_700MHz_A | R4 | RP-212618 | Puloli |
| 930157 | Core part: Introduction of upper 700MHz A block E-UTRA band for the US | LTE_upper_700MHz_A-Core | R4 | RP-212618 | Puloli |
| 930257 | Perf. part: Introduction of upper 700MHz A block E-UTRA band for the US | LTE_upper_700MHz_A-Perf | R4 | RP-212618 | Puloli |
| 930058 | High power UE (power class 2) for one NR FDD band | NR_PC2_UE_FDD | R4 | RP-212633 | China Unicom |
| 930158 | Core part: High power UE (power class 2) for one NR FDD band | NR_PC2_UE_FDD-Core | R4 | RP-212633 | China Unicom |
| 930258 | Perf. part: High power UE (power class 2) for one NR FDD band | NR_PC2_UE_FDD-Perf | R4 | RP-212633 | China Unicom |

Summary based on the input provided by XXX in xP-21xxxx.

# 17	NR Carrier Aggregation

## 17.1	NR intra band Carrier Aggregation (itrabCA)

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 881005 | NR itrabCA for xCC DL/yCC UL including contiguous and non-contiguous spectrum (x>=y) | NR_CA_R17_Intra |  | RP-211757 | Ericsson |
| 881105 | Core part: NR_CA_R17_Intra | NR_CA_R17_Intra-Core | R4 | RP-211757 | Ericsson |
| 881205 | Perf. Part: NR_CA_R17_Intra | NR_CA_R17_Intra-Perf | R4 | RP-211757 | Ericsson |

Summary based on the input provided by XXX in xP-21xxxx.

## 17.2	NR inter band Carrier Aggregation (iterbCA)

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 900063 | High power UE for NR TDD intra-band Carrier Aggregation in frequency range FR1 | NR_intra_HPUE_R17 |  | RP-212180 | Huawei |
| 900163 | Core part: NR_intra_HPUE_R17 | NR_intra_HPUE_R17-Core | R4 | RP-212180 | Huawei |
| 900263 | Perf. Part: NR_intra_HPUE_R17 | NR_intra_HPUE_R17-Perf | R4 | RP-212180 | Huawei |
| 881006 | NR iterbCA/Dual Connectivity for 2 bands DL with x bands UL (x=1,2) | NR_CADC_R17_2BDL_xBUL |  | RP-212511 | ZTE |
| 881106 | Core part: NR_CADC_R17_2BDL_xBUL | NR_CADC_R17_2BDL_xBUL-Core | R4 | RP-212511 | ZTE |
| 881206 | Perf. part: NR_CADC_R17_2BDL_xBUL | NR_CADC_R17_2BDL_xBUL-Perf | R4 | RP-212511 | ZTE |
| 881007 | NR iterbCA for 3 bands DL with 1 band UL | NR_CA_R17_3BDL_1BUL |  | RP-212239 | CATT |
| 881107 | Core part: NR_CA_R17_3BDL_1BUL | NR_CA_R17_3BDL_1BUL-Core | R4 | RP-212239 | CATT |
| 881207 | Perf. part: NR_CA_R17_3BDL_1BUL | NR_CA_R17_3BDL_1BUL-Perf | R4 | RP-212239 | CATT |
| 881008 | NR iterbCA/Dual Connectivity for 3 bands DL with 2 bands UL | NR_CADC_R17_3BDL_2BUL |  | RP-212512 | ZTE |
| 881108 | Core part: NR_CADC_R17_3BDL_2BUL | NR_CADC_R17_3BDL_2BUL-Core | R4 | RP-212512 | ZTE |
| 881208 | Perf. part: NR_CADC_R17_3BDL_2BUL | NR_CADC_R17_3BDL_2BUL-Perf | R4 | RP-212512 | ZTE |
| 881009 | NR iterbCA for 4 bands DL with 1 band UL | NR_CA_R17_4BDL_1BUL |  | RP-211759 | Ericsson |
| 881109 | Core part: NR_CA_R17_4BDL_1BUL | NR_CA_R17_4BDL_1BUL-Core | R4 | RP-211759 | Ericsson |
| 881209 | Perf. part: NR_CA_R17_4BDL_1BUL | NR_CA_R17_4BDL_1BUL-Perf | R4 | RP-211759 | Ericsson |
| 881010 | NR iterbCA/Dual connectivity for DL 4 bands and 2UL bands | NR_CADC_R17_4BDL_2BUL |  | RP-212110 | Samsung |
| 881110 | Core part: NR_CADC_R17_4BDL_2BUL | NR_CADC_R17_4BDL_2BUL-Core | R4 | RP-212110 | Samsung |
| 881210 | Perf. part: NR_CADC_R17_4BDL_2BUL | NR_CADC_R17_4BDL_2BUL-Perf | R4 | RP-212110 | Samsung |
| 881011 | NR iterbCA for 5 bands DL with x bands UL (x=1, 2) | NR_CADC_R17_5BDL_xBUL |  | RP-212176 | Huawei |
| 881111 | Core part: NR_CADC_R17_5BDL_xBUL | NR_CADC_R17_5BDL_xBUL-Core | R4 | RP-212176 | Huawei |
| 881211 | Perf. part: NR_CADC_R17_5BDL_xBUL | NR_CADC_R17_5BDL_xBUL-Perf | R4 | RP-212176 | Huawei |
| 890054 | Rel-17 High power UE for NR inter-band Carrier Aggregation with 2 bands downlink and x bands uplink (x=1,2) | NR_PC2_CA_R17_2BDL_2BUL |  | RP-211831 | China Telecom |
| 890154 | Core part: NR_PC2_CA_R17_2BDL_2BUL | NR_PC2_CA_R17_2BDL_2BUL-Core | R4 | RP-211831 | China Telecom |
| 890254 | Perf. part: NR_PC2_CA_R17_2BDL_2BUL | NR_PC2_CA_R17_2BDL_2BUL-Perf | R4 | RP-211831 | China Telecom |
| 920066 | UE Conformance - Rel-17 High power UE for NR inter-band Carrier Aggregation with 2 bands downlink and x bands uplink (x=1,2) | NR_PC2_CA_R17_2BDL_2BUL-UEConTest | R5 | RP-211140 | China Telecom |

Summary based on the input provided by XXX in xP-21xxxx.

# 18	NR and LTE

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 920072 | LTE/NR spectrum sharing in Band 34/n34 and Band 39/n39 | DSS_LTE_B34_NR_Bn34_LTE_B39_NR_Bn39 | R4 |  | CMCC |
| 920172 | Core part: LTE/NR spectrum sharing in Band 34/n34 and Band 39/n39 | DSS_LTE_B34_NR_Bn34_LTE_B39_NR_Bn39-Core | R4 | RP-211996 | CMCC |
| 920073 | High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands | LTE_NR_HPUE_FWVM | R4 | RP-212533 | Nokia |
| 920173 | Core part: High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands | LTE_NR_HPUE_FWVM-Core | R4 | RP-212533 | Nokia |
| 920273 | Perf. part: High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands | LTE_NR_HPUE_FWVM-Perf | R4 | RP-212533 | Nokia |
| 920074 | Rel-17 Dual Connectivity (DC) of x bands (x=1,2) LTE inter-band CA (xDL1UL) and 4 bands NR inter-band CA (4DL1UL) | DC_R17_xBLTE_4BNR_yDL2UL | R4 | RP-212606 | Huawei |
| 920174 | Core part: Rel-17 Dual Connectivity (DC) of x bands (x=1,2) LTE inter-band CA (xDL1UL) and 4 bands NR inter-band CA (4DL1UL) | DC_R17_xBLTE_4BNR_yDL2UL-Core | R4 | RP-212606 | Huawei |
| 920274 | Perf. part: Rel-17 Dual Connectivity (DC) of x bands (x=1,2) LTE inter-band CA (xDL1UL) and 4 bands NR inter-band CA (4DL1UL) | DC_R17_xBLTE_4BNR_yDL2UL-Perf | R4 | RP-212606 | Huawei |
| 911019 | Addition of MSD (Maximum Sensitivity Degradation) for inter-band EN-DC combinations (1 band LTE+1 band NR FR1) due to added channel bandwidths | ENDC_R17_1BLTE_1BNR_MS |  | RP-212187 | Huawei, HiSilicon |
| 911119 | Core part: ENDC_R17_1BLTE_1BNR_MS | ENDC_R17_1BLTE_1BNR_MSD-Core |  | RP-212187 | Huawei, HiSilicon |
| 911011 | Rel-17 Power Class 2 for EN-DC with x LTE bands + y NR band(s) in DL and with 1 LTE band +1 TDD NR band in UL (either x= 2, 3, y=1 or x=1, 2, y=2) | ENDC_PC2_R17_xLTE_yNR |  | RP-211760 | Ericsson |
| 911111 | Core part: ENDC_PC2_R17_xLTE_yNR | ENDC_PC2_R17_xLTE_yNR-Core | R4 | RP-211760 | Ericsson |
| 911211 | Perf. Part: ENDC_PC2_R17_xLTE_yNR | ENDC_PC2_R17_xLTE_yNR-Perf | R4 | RP-211760 | Ericsson |
| 930051 | UE Conformance - Power Class 2 for EN-DC with x LTE bands + y NR band(s) in DL and with 1 LTE band +1 TDD NR band in UL (either x= 2, 3, y=1 or x=1, 2, y=2) | ENDC_PC2_R17_xLTE_yNR-UEConTest | R5 | RP-211870 | Ericsson |
| 911018 | Simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC | LTE_NR_Simult_RxTx |  | RP-211382 | Huawei |
| 911118 | Core part: LTE_NR_Simult_RxTx | LTE_NR_Simult_RxTx-Core | R4 | RP-211382 | Huawei |
| 911218 | Perf. part: LTE_NR_Simult_RxTx | LTE_NR_Simult_RxTx-Perf | R4 | RP-211382 | Huawei |
| 890052 | Rel-17 Dual Connectivity (DC) of 5 bands LTE inter-band CA (5DL/1UL) and 1 NR band (1DL/1UL) | DC_R17_5BLTE_1BNR_6DL2UL |  | RP-211195 | Samsung |
| 890152 | Core part: DC_R17_5BLTE_1BNR_6DL2UL | DC_R17_5BLTE_1BNR_6DL2UL-Core | R4 | RP-211195 | Samsung |
| 890252 | Perf. part: DC_R17_5BLTE_1BNR_6DL2UL | DC_R17_5BLTE_1BNR_6DL2UL-Perf | R4 | RP-211195 | Samsung |
| 890053 | Rel-17 Dual Connectivity (DC) of x bands (x=2, 3, 4) LTE inter-band CA (xDL/1UL) and 1 NR FR1 band (1DL/1UL) and 1 NR FR2 band (1DL/1UL) | DC_R17_xBLTE_2BNR_yDL3UL |  | RP-202542 | Samsung |
| 890153 | Core part: DC_R17_xBLTE_2BNR_yDL3UL | DC_R17_xBLTE_2BNR_yDL3UL-Core | R4 | RP-202542 | Samsung |
| 890253 | Perf. part: DC_R17_xBLTE_2BNR_yDL3UL | DC_R17_xBLTE_2BNR_yDL3UL-Perf | R4 | RP-202542 | Samsung |
| 880080 | Band combinations for concurrent operation of NR/LTE Uu bands/band combinations and one NR/LTE V2X PC5 band | NR_LTE_V2X_PC5_combos |  | RP-210588 | CATT |
| 880180 | Core part: NR_LTE_V2X_PC5_combos | NR_LTE_V2X_PC5_combos-Core | R4 | RP-210588 | CATT |
| 880280 | Perf. part: NR_LTE_V2X_PC5_combos | NR_LTE_V2X_PC5_combos-Perf | R4 | RP-210588 | CATT |
| 880094 | High power UE (power class 2) for EN-DC with 1 LTE band + 1 NR TDD band | ENDC_UE_PC2_R17_NR_TDD |  | RP-211853 | China Unicom |
| 880194 | Core part: ENDC_UE_PC2_R17_NR_TDD | ENDC_UE_PC2_R17_NR_TDD-Core | R4 | RP-211853 | China Unicom |
| 880294 | Perf. part: ENDC_UE_PC2_R17_NR_TDD | ENDC_UE_PC2_R17_NR_TDD-Perf | R4 | RP-211853 | China Unicom |
| 911000 | UE Conformance - ENDC_UE_PC2_R17_NR_TDD | ENDC_UE_PC2_R17_NR_TDD-UEConTest | R5 | RP-211420 | China Unicom |
| 880095 | LTE/NR spectrum sharing in Band 38/n38 | DSS_LTE_B38_NR_Bn38 |  | RP-201314 | Vodafone |
| 880195 | Core part: DSS_LTE_B38_NR_Bn38 | DSS_LTE_B38_NR_Bn38-Core | R4 | RP-201314 | Vodafone |
| 880096 | LTE/NR spectrum sharing in Band 40/n40 | DSS_LTE_B40_NR_Bn40 |  | RP-202084 | Reliance Jio |
| 880196 | Core part: DSS_LTE_B40_NR_Bn40 | DSS_LTE_B40_NR_Bn40-Core | R4 | RP-202084 | Reliance Jio |
| 880296 | Perf. part: DSS_LTE_B40_NR_Bn40 | DSS_LTE_B40_NR_Bn40-Perf | R4 | RP-202084 | Reliance Jio |
| 880098 | Dual Connectivity (DC) of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL) | DC_R17_1BLTE_1BNR_2DL2UL |  | RP-212096 | CHTTL |
| 880198 | Core part: DC_R17_1BLTE_1BNR_2DL2UL | DC_R17_1BLTE_1BNR_2DL2UL-Core | R4 | RP-212096 | CHTTL |
| 880298 | Perf. part: DC_R17_1BLTE_1BNR_2DL2UL | DC_R17_1BLTE_1BNR_2DL2UL-Perf | R4 | RP-212096 | CHTTL |
| 880099 | Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL) | DC_R17_2BLTE_1BNR_3DL2UL |  | RP-212174 | Huawei |
| 880199 | Core part: DC_R17_2BLTE_1BNR_3DL2UL | DC_R17_2BLTE_1BNR_3DL2UL-Core | R4 | RP-212174 | Huawei |
| 890263 | Perf. part: DC_R17_2BLTE_1BNR_3DL2UL | DC_R17_2BLTE_1BNR_3DL2UL-Perf | R4 | RP-212174 | Huawei |
| 881000 | Dual Connectivity (DC) of 3 bands LTE inter-band CA (3DL/1UL) and 1 NR band (1DL/1UL) | DC_R17_3BLTE_1BNR_4DL2UL |  | RP-211758 | Ericsson |
| 881100 | Core part: DC_R17_3BLTE_1BNR_4DL2UL | DC_R17_3BLTE_1BNR_4DL2UL-Core | R4 | RP-211758 | Ericsson |
| 881200 | Perf. part: DC_R17_3BLTE_1BNR_4DL2UL | DC_R17_3BLTE_1BNR_4DL2UL-Perf | R4 | RP-211758 | Ericsson |
| 881001 | Dual Connectivity (DC) of 4 bands LTE inter-band CA (4DL/1UL) and 1 NR band (1DL/1UL) | DC_R17_4BLTE_1BNR_5DL2UL |  | RP-212532 | Nokia |
| 881101 | Core part: DC_R17_4BLTE_1BNR_5DL2UL | DC_R17_4BLTE_1BNR_5DL2UL-Core | R4 | RP-212532 | Nokia |
| 881201 | Perf. part: DC_R17_4BLTE_1BNR_5DL2UL | DC_R17_4BLTE_1BNR_5DL2UL-Perf | R4 | RP-212532 | Nokia |
| 881002 | Dual Connectivity (DC) of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) | DC_R17_xBLTE_2BNR_yDL2UL |  | RP-211750 | LG Electronics |
| 881102 | Core part: DC_R17_xBLTE_2BNR_yDL2UL | DC_R17_xBLTE_2BNR_yDL2UL-Core | R4 | RP-211750 | LG Electronics |
| 881202 | Perf. part: DC_R17_xBLTE_2BNR_yDL2UL | DC_R17_xBLTE_2BNR_yDL2UL-Perf | R4 | RP-211750 | LG Electronics |
| 881003 | Dual Connectivity (DC) of x bands (x=1,2) LTE inter-band CA (xDL/xUL) and y bands (y=3-x) NR inter-band CA (yDL/yUL) | DC_R17_xBLTE_yBNR_3DL3UL |  | RP-211803 | ZTE |
| 881103 | Core part: DC_R17_xBLTE_yBNR_3DL3UL | DC_R17_xBLTE_yBNR_3DL3UL-Core | R4 | RP-211803 | ZTE |
| 881203 | Perf. part: DC_R17_xBLTE_yBNR_3DL3UL | DC_R17_xBLTE_yBNR_3DL3UL-Perf | R4 | RP-211803 | ZTE |
| 881004 | Dual Connectivity (DC) of x bands (x=1,2,3) LTE inter-band CA (xDL/1UL) and 3 bands NR inter-band CA (3DL/1UL) | DC_R17_xBLTE_3BNR_yDL2UL |  | RP-211805 | ZTE |
| 881104 | Core part: DC_R17_xBLTE_3BNR_yDL2UL | DC_R17_xBLTE_3BNR_yDL2UL-Core | R4 | RP-211805 | ZTE |
| 881204 | Perf. part: DC_R17_xBLTE_3BNR_yDL2UL | DC_R17_xBLTE_3BNR_yDL2UL-Perf | R4 | RP-211805 | ZTE |
| 881012 | band combinations for SA NR supplementary uplink (SUL), NSA NR SUL, NSA NR SUL with UL sharing from the UE perspective (ULSUP) | NR_SUL_combos_R17 |  | RP-212178 | Huawei |
| 881112 | Core part: NR_SUL_combos_R17 | NR_SUL_combos_R17-Core | R4 | RP-212178 | Huawei |
| 881212 | Perf. part: NR_SUL_combos_R17 | NR_SUL_combos_R17-Perf | R4 | RP-212178 | Huawei |
| 900056 | UE Conformance - Rel-17 NR CA and DC; and NR and LTE DC Configurations | NR_CADC_NR_LTE_DC_R17-UEConTest | R5 | RP-211357 | Huawei |

Summary based on the input provided by XXX in xP-21xxxx.

# 19	NR new/modified bands

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 911016 | Introduction of 900MHz NR band for Europe for Rail Mobile Radio (RMR) | NR_RAIL_EU_900MHz |  | RP-211495 | UIC |
| 911116 | Core part: NR_RAIL_EU_900MHz | NR_RAIL_EU_900MHz-Core | R4 | RP-211495 | UIC |
| 911216 | Perf. part: NR_RAIL_EU_900MHz | NR_RAIL_EU_900MHz-Perf | R4 | RP-211495 | UIC |
| 911017 | Introduction of 1900MHz NR TDD band for Europe for Rail Mobile Radio (RMR) | NR_RAIL_EU_1900MHz_TDD |  | RP-211542 | UIC |
| 911117 | Core part: NR_RAIL_EU_1900MHz_TDD | NR_RAIL_EU_1900MHz_TDD-Core | R4 | RP-211542 | UIC |
| 911217 | Perf. part: NR_RAIL_EU_1900MHz_TDD | NR_RAIL_EU_1900MHz_TDD-Perf | R4 | RP-211542 | UIC |
| 860041 | Extending current NR operation to 71GHz | NR_ext_to_71GHz | R1 | RP-212637 | Qualcomm |
| 860141 | Core part: Extending current NR operation to 71GHz | NR_ext_to_71GHz-Core | R1 | RP-212637 | Qualcomm |
| 860241 | Perf. part: Extending current NR operation to 71GHz | NR_ext_to_71GHz-Perf | R4 | RP-212637 | Qualcomm |
| 860037 | Study on supporting NR from 52.6 GHz to 71 GHz | FS_NR_52_to_71GHz | R1 | RP-201838 | Intel |
| 870060 | Study on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz | FS_NR_IMT_param | R4 | RP-202276 | Ericsson |
| 870063 | Introduction of NR band n13 | NR_n13 | R4 | RP-200480 | Huawei |
| 870163 | Core part: Introduction of NR band n13 | NR_n13-Core | R4 | RP-200480 | Huawei |
| 870263 | Perf. part: Introduction of NR band n13 | NR_n13-Perf | R4 | RP-200480 | Huawei |
| 880082 | Introduction of NR 47GHz band | NR_47GHz_band | R4 | RP-212528 | Ericsson |
| 880182 | Core part: Introduction of NR 47GHz band | NR_47GHz_band-Core | R4 | RP-212528 | Ericsson |
| 880282 | Perf. part: Introduction of NR 47GHz band | NR_47GHz_band-Perf | R4 | RP-212528 | Ericsson |
| 880084 | Introduction of NR band 24 | NR_band_n24 | R4 | RP-211505 | Ligado Networks |
| 880184 | Core part: Introduction of NR band 24 | NR_band_n24-Core | R4 | RP-211505 | Ligado Networks |
| 880284 | Perf. part: Introduction of NR band 24 | NR_band_n24-Perf | R4 | RP-211505 | Ligado Networks |
| 890050 | Introduction of 6GHz NR licensed bands | NR_6GHz | R4 | RP-202844 | Huawei |
| 890150 | Core part: Introduction of 6GHz NR licensed bands | NR_6GHz-Core | R4 | RP-202844 | Huawei |
| 890250 | Perf. part: Introduction of 6GHz NR licensed bands | NR_6GHz-Perf | R4 | RP-202844 | Huawei |
| 920075 | Introduction of operation in full unlicensed band 5925-7125MHz for NR | NR_6GHz_unlic_full | R4 | RP-212302 | Apple |
| 920175 | Core part: Introduction of operation in full unlicensed band 5925-7125MHz for NR | NR_6GHz_unlic_full-Core | R4 | RP-212302 | Apple |
| 920275 | Perf. part: Introduction of operation in full unlicensed band 5925-7125MHz for NR | NR_6GHz_unlic_full-Perf | R4 | RP-212302 | Apple |
| 890051 | Introduction of lower 6GHz NR unlicensed operation for Europe | NR_6GHz_unlic_EU | R4 | RP-212625 | Nokia |
| 890151 | Core part: Introduction of lower 6GHz NR unlicensed operation for Europe | NR_6GHz_unlic_EU-Core | R4 | RP-212625 | Nokia |
| 890251 | Perf. part: Introduction of lower 6GHz NR unlicensed operation for Europe | NR_6GHz_unlic_EU-Perf | R4 | RP-212625 | Nokia |
| 900065 | Introduction of NR band n67 | NR_n67 | R4 | RP-202829 | Ericsson |
| 900165 | Core part: Introduction of NR band n67 | NR_n67-Core | R4 | RP-202829 | Ericsson |
| 900265 | Perf. Part: Introduction of NR band n67 | NR_n67-Perf | R4 | RP-202829 | Ericsson |
| 900066 | Introduction of NR band n85 | NR_n85 | R4 | RP-210707 | Ericsson |
| 900166 | Core part: Introduction of NR band n85 | NR_n85-Core | R4 | RP-210707 | Ericsson |
| 900266 | Perf. part: Introduction of NR band n85 | NR_n85-Perf | R4 | RP-210707 | Ericsson |
| 880083 | Introduction of 1.6 GHz NR supplemental uplink (SUL) band with same uplink frequency range of Band 24 | NR_SUL_UL_n24 |  | RP-210341 | Ligado Networks |
| 880183 | Core part: NR_SUL_UL_n24 | NR_SUL_UL_n24-Core | R4 | RP-210341 | Ligado Networks |
| 880283 | Perf. part: NR_SUL_UL_n24 | NR_SUL_UL_n24-Perf | R4 | RP-210341 | Ligado Networks |
| 880085 | Introduction of NR supplemental uplink (SUL) band 1880-1920MHz | NR_SUL_band_1880_1920MHz |  | RP-201363 | CMCC |
| 880185 | Core part: NR_SUL_band_1880_1920MHz | NR_SUL_band_1880_1920MHz-Core | R4 | RP-201363 | CMCC |
| 880285 | Perf. part: NR_SUL_band_1880_1920MHz | NR_SUL_band_1880_1920MHz-Perf | R4 | RP-201363 | CMCC |
| 880086 | Introduction of NR supplemental uplink (SUL) band 2300-2400MHz | NR_SUL_band_2300_2400MHz |  | RP-201364 | CMCC |
| 880186 | Core part: NR_SUL_band_2300_2400MHz | NR_SUL_band_2300_2400MHz-Core | R4 | RP-201364 | CMCC |
| 880286 | Perf. part: NR_SUL_band_2300_2400MHz | NR_SUL_band_2300_2400MHz-Perf | R4 | RP-201364 | CMCC |
| 900055 | UE Conformance - New Rel-17 NR licensed bands and extension of existing NR bands | NR_lic_bands_BW_R17-UEConTest | R5 | RP-202567 | Huawei |

Summary based on the input provided by XXX in xP-21xxxx.

# 20	LTE new/modified bands

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 911020 | New bands and bandwidth allocation for 5G terrestrial broadcast - part 1 | LTE_terr_bcast_bands_part1 | R1 | RP-211144 | EBU |
| 911120 | New bands and bandwidth allocation for 5G terrestrial broadcast - part 1 | LTE_terr_bcast_bands_part1 | R1 | RP-211144 | EBU |
| 880079 | Additional LTE bands for UE categories M1/M2/NB1/NB2 | LTE_bands_R17_M1_M2_NB1_NB2 |  | RP-211857 | Ericsson |
| 880179 | Core part: LTE_bands_R17_M1_M2_NB1_NB2 | LTE_bands_R17_M1_M2_NB1_NB2-Core | R4 | RP-211857 | Ericsson |
| 880279 | Perf. part: LTE_bands_R17_M1_M2_NB1_NB2 | LTE_bands_R17_M1_M2_NB1_NB2-Perf | R4 | RP-211857 | Ericsson |
| 880081 | Modification of LTE Band 24 Specifications to comply with updated regulatory emission limits | LTE_B24_mod |  | RP-211507 | Ligado Networks |
| 880181 | Core part: LTE_B24_mod | LTE_B24_mod-Core | R4 | RP-211507 | Ligado Networks |
| 880281 | Perf. part: LTE_B24_mod | LTE_B24_mod-Perf | R4 | RP-211507 | Ligado Networks |
| 920067 | UE Conformance - Modification of LTE Band 24 Specifications to comply with updated regulatory emission limits | LTE_B24_mod-UEConTest | R5 | RP-212497 | Ligado Networks |

Summary based on the input provided by XXX in xP-21xxxx.

# 21	LTE inter-band Carrier Aggregation (LTE iterbCA)

| UID | Name | Acronym | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 880087 | LTE iterbCA for 2 bands DL with 1 band UL | LTE_CA_R17_2BDL_1BUL |  | RP-211859 | Qualcomm |
| 880187 | Core part: LTE_CA_R17_2BDL_1BUL | LTE_CA_R17_2BDL_1BUL-Core | R4 | RP-211859 | Qualcomm |
| 880287 | Perf. part: LTE_CA_R17_2BDL_1BUL | LTE_CA_R17_2BDL_1BUL-Perf | R4 | RP-211859 | Qualcomm |
| 880088 | LTE iterbCA for 3 bands DL with 1 band UL | LTE_CA_R17_3BDL_1BUL |  | RP-212192 | Huawei |
| 880188 | Core part: LTE_CA_R17_3BDL_1BUL | LTE_CA_R17_3BDL_1BUL-Core | R4 | RP-212192 | Huawei |
| 880288 | Stopped - Perf. part: LTE iterbCA for 3 bands DL with 1 band UL | LTE_CA_R17_3BDL_1BUL-Perf | R4 | RP-202615 | Huawei |
| 880089 | LTE iterbCA Core for x bands DL (x=4, 5, 6) with 1 band UL | LTE_CA_R17_xBDL_1BUL |  | RP-211885 | Nokia |
| 880189 | Core part: LTE_CA_R17_xBDL_1BUL | LTE_CA_R17_xBDL_1BUL-Core | R4 | RP-211885 | Nokia |
| 880289 | Perf. part: LTE_CA_R17_xBDL_1BUL | LTE_CA_R17_xBDL_1BUL-Perf | R4 | RP-211885 | Nokia |
| 880090 | LTE iterbCA for 2 bands DL with 2 bands UL | LTE_CA_R17_2BDL_2BUL |  | RP-212194 | Huawei |
| 880190 | Core part: LTE_CA_R17_2BDL_2BUL | LTE_CA_R17_2BDL_2BUL-Core | R4 | RP-212194 | Huawei |
| 880290 | Perf. part: LTE_CA_R17_2BDL_2BUL | LTE_CA_R17_2BDL_2BUL-Perf | R4 | RP-212194 | Huawei |
| 880091 | LTE iterbCA for x bands DL (x= 3, 4, 5) with 2 bands UL | LTE_CA_R17_xBDL_2BUL |  | RP-211748 | LG |
| 880191 | Core part: LTE_CA_R17_xBDL_2BUL | LTE_CA_R17_xBDL_2BUL-Core | R4 | RP-211748 | LG |
| 880291 | Perf. part: LTE_CA_R17_xBDL_2BUL | LTE_CA_R17_xBDL_2BUL-Perf | R4 | RP-211748 | LG |

Summary based on the input provided by XXX in xP-21xxxx.

# 22	Other Release 17 Features

| 911037 | Reliable Data Service Serialization Indication | RDSSI |  | SP-190446 | Starsinic, Michael, Convida Wireless LLC |
| --- | --- | --- | --- | --- | --- |
| 840017 | Stage 2 of RDSSI | RDSSI | S2 | SP-190446 | Starsinic, Michael, Convida Wireless LLC |
| 900005 | CT aspects of RDSSI | RDSSI | C1 | CP-203234 | Vivek Gupta, Intel |
| 930002 | IMS voice service support and network usability guarantee for UE s E-UTRA capability disabled scenario in SA 5GS | ING_5GS | ct | CP-212231 | Mingxue Li, China Telecom |
| 930047 | CT1 aspects of ING_5GS | ING_5GS | C1 | CP-212231 | Mingxue Li, China Telecom |
| 930046 | CT6 aspects of ING_5GS | ING_5GS | C6 | CP-212231 | Mingxue Li, China Telecom |
| 930031 | User Consent for 3GPP services | FS_UC3S | S3 | SP-200885 | Rong Wu, Huawei Technologies |
| 890037 | Study on User Consent for 3GPP services | FS_UC3S | S3 | SP-200885 | Rong Wu, Huawei Technologies |
| 930006 | Security aspects on User Consent for 3GPP services | UC3S_SEC | S3 | SP-210836 | Rong Wu, Huawei Technologies |
| 930032 | Improved support for NSA in the service-based management architecture | NSA_SBMA | S5 | SP-211121 | Lan Zou, Huawei |
| 910031 | Study on Enhancement of service-based management architecture | FS_eSBMA | S5 | SP-210136 | Lan Zou, Huawei |
| 930009 | Improved support for NSA in the service-based management architecture | NSA_SBMA | S5 | SP-210858 | Lan Zou, Huawei |
| 930033 | Access control for management service | MSAC | S5 | SP-210859 | Jing Ping, Nokia |
| 890016 | Study on access control for management service | FS_MNSAC | S5 | SP-200853 | Jing Ping, Nokia |
| 930010 | Access control for management service | MSAC | S5 | SP-210859 | Jing Ping, Nokia |
| 920060 | NPN support of PWS | NPN_PWS | S1 | SP-210585 | Francesco Pica, Qualcomm |
| 910007 | Start of Pause of Charging via User Plane | SPOCUP | C4 | CP-210076 | YONG, YANG, Ericsson, frank |
| 910030 | File Management | FIMA | S5 | SP-210135 | Pollakowski, Olaf, Nokia |
| 910024 | Enhancements of 3GPP profiles for cryptographic algorithms and security protocols | eCryptPr | S3 | SP-210107 | Pinar Comak, Ericsson |
| 920027 | AKMA TLS protocol profiles | AKMA_TLS | S3 | SP-210424 | Escott, Adrian, Qualcomm |
| 910025 | User Plane Integrity Protection for LTE | UPIP_SEC_LTE | S3 | SP-210105 | Evans, Tim, Vodafone |
| 900002 | Restoration of PDN Connections in PGW-C/SMF Set | RPCPSET | C4 | CP-203020 | LANDAIS, Bruno, Nokia |
| 900037 | Enhancement of Inter-PLMN Roaming | EoIPR | C4 | CP-203021 | Caixia Qi, Huawei |
| 900021 | Management Aspects of 5G Network Sharing | MANS | S5 | SP-201080 | Mingrui Sun, China Unicom |
| 920018 | Study on Management Aspects of 5G Network Sharing | FS_MANS | S5 | SP-210387 | Mingrui Sun, China Unicom |
| 900019 | Adapting BEST for use in 5G networks | BEST_5G | S3 | SP-201020 | Keesmaat, Iko, KPN |
| 890011 | Mission critical security enhancements phase 2 | MCXSec2 | S3 | SP-200879 | Woodward, Tim, Motorola Solutions, Inc, |
| 890070 | PAP/CHAP protocols usage in 5GS | PAP_CHAP | ct | CP-202251 | Mingxue Li, China Telecom |
| 890071 | CT1 aspects of PAP/CHAP protocols usage in 5GS | PAP_CHAP | C1 | CP-202251 | Mingxue Li, China Telecom |
| 890072 | CT3 aspects of PAP/CHAP protocols usage in 5GS | PAP_CHAP | C3 | CP-210188 | Mingxue Li, China Telecom |
| 890001 | Service-based support for SMS in 5GC | SMS_SBI |  | CP-212023 | HAOUARI, Wafa, Orange |
| 890028 | CT1 aspects of SMS_SBI | SMS_SBI | C1 | CP-212023 | HAOUARI, Wafa, Orange |
| 890029 | CT4 aspects of SMS_SBI | SMS_SBI | C4 | CP-212023 | HAOUARI, Wafa, Orange |
| 840048 | Enhanced Relays for Energy eFficiency and Extensive Coverage | REFEC |  | SP-190307 | Norp, Toon, KPN |
| 810018 | Study on REFEC | FS_REFEC | S1 | SP-180785 | Jose Luis Almodovar Chico, KPN |
| 840034 | Stage 1 of REFEC | REFEC | S1 | SP-190307 | Norp, Toon, KPN |
| 840044 | Network Controlled Interactive Service | NCIS |  | SP-190303 | Ning YANG, oppo |
| 800015 | Study on NCIS | FS_NCIS | S1 | SP-180341 | YANG Ning, Oppo |
| 840030 | Stage 1 of NCIS | NCIS | S1 | SP-191039 | Ning YANG, oppo |
| 840019 | 5G System Enhancement for Advanced Interactive Services | 5G_AIS | S2 | SP-190564 | Lei Yixue, Tencent |
| 890009 | Points for 8K VR 360 Video over 5G | 8K_VR_5G | S4 | SP-200667 | Ozgur Oyman, Company: Intel |
| 880043 | Multi-Device and multi-identity Enhancements (MuDE) | MuDE |  | SP-190823 | Adrian.buckley@vivo.com |
| 850041 | Stage 1 of MuDE | MuDE | S1 | SP-190823 | Adrian.buckley@vivo.com |
| 880018 | Stage 3 of MuDE | MuDe | C1 | CP-201162 | Adrian Buckley |
| 920055 | Best Practice of Packet Forwarding Control Protocol (PFCP) | BEPoP | ct | CP-212024 | Song Yue, China Mobile |
| 920056 | CT3 aspects of BEPoP | BEPoP | C3 | CP-212024 | Song Yue, China Mobile |
| 880014 | CT4 aspects of BEPoP | BEPoP | C4 | CP-212024 | Song Yue, China Mobile |
| 880029 | Enhancement of Handover Optimization | E_HOO | S5 | SP-200466 | Elmdahl, Per, Ericsson |
| 880028 | Management data collection control and discovery | MADCOL | S5 | SP-200465 | Pollakowski, Olaf, Nokia |
| 880026 | Management of the enhanced tenant concept | eMEMTANE | S5 | SP-200463 | Zhu, Lei, Huawei |
| 880025 | Enhancements of 5G performance measurements and KPIs | ePM_KPI_5G | S5 | SP-200462 | Yizhi Yao, Intel |
| 880012 | Extension for headset interface tests of UE | HInT | S4 | SP-200398 | Jan Reimes, HEAD acoustics GmbH, |
| 840038 | MC services support on IOPS mode of operation | MCIOPS | S6 | SP-190944 | Camilo.solano@ericsson.com |
| 840016 | Supporting Flexible Local Area Data Network | FLADN | S2 | SP-190563 | LG Electronics, Hyunsook Kim |
| 820003 | Support of Immersive Teleconferencing and Telepresence for Remote Terminals | ITT4RT | S4 | SP-180985 | Ozgur Oyman, Intel |
| 830005 | Terminal Audio quality performance and Test methods for Immersive Audio Services | ATIAS | S4 | SP-190040 | Stéphane Ragot, Orange |
| 860012 | Handsets Featuring Non-Traditional Earpieces | HaNTE | S4 | SP-190989 | Schevciw, Andre, Qualcomm |
| 920008 | 5GMS AF Event Exposure | EVEX | S4 | SP-210374 | Lo, Charles, Qualcomm Inc., |
| 920009 | Edge Extensions to the 5G Media Streaming Architecture | 5GMS_EDGE | S4 | SP-210375 | Bouazizi, Imed, Qualcomm Inc. |
| 920010 | 5G Multicast-Broadcast User Service Architecture and related 5GMS Extensions | 5MBUSA | S4 | SP-210376 | TAN, PENG, TELUS |
| 920011 | 8K Television over 5G | 8K_TV_5G | S4 | SP-210381 | Thomas Stockhammer |
| 830017 | Verification-modified Calling Name Display | VMOD_DISPLAY | S1 | SP-190085 | Hala Mowafy, Ericsson |
| 850038 | Enhanced Calling Name Service Analytics Interworking | eCNAM_An | S1 | SP-190940 | Hala.Mowafy@ericsson.com |
| 880049 | Enhancement for the 5G Control Plane Steering of Roaming for UE in Connected mode | eCPSOR_CON |  | CP-210148 | Minokuchi, Atsushi, NTT DOCOMO |
| 850039 | Stage 1 of eCPSOR_CON | eCPSOR_CON | S1 | SP-190941 | Minokuchi, Atsushi, NTT DOCOMO |
| 880022 | Stage 3 (CT1) of eCPSOR_CON | eCPSOR_CON | C1 | CP-210148 | Al-Bakri Ban, NTT DOCOMO |
| 900036 | Stage 3 (CT4) of eCPSOR_CON | eCPSOR_CON | C4 | CP-210148 | Al-Bakri Ban, NTT DOCOMO |
| 920061 | Stage 3 (CT6) of eCPSOR_CON | eCPSOR_CON | C6 | CP-210148 | Al-Bakri Ban, NTT DOCOMO |
| 910090 | Integration of Generic Bootstrapping Architecture (GBA) into 5GC | GBA_5G |  | SP-190714 | Vlasios Tsiatsis, Ericsson |
| 850023 | Security aspects of Integration of GBA into 5GC | GBA_5G | S3 | SP-190714 | Vlasios Tsiatsis, Ericsson |
| 910004 | CT aspects of Integration of GBA into SBA | GBA_5G | C4 | CP-210283 | de Gregorio, Jesús (Ericsson) |
| 910047 | Integration of Generic Bootstrapping Architecture (GBA) into 5GC | GBA_5G | S3 | SP-190714 | Vlasios Tsiatsis, Ericsson |
| 770024 | EVS Codec Extension for Immersive Voice and Audio Services | IVAS_Codec | S4 | SP-170611 | Bin Wang, Huawei Technologies Co Ltd |
| 860008 | IMS emergency support for Stand-alone Non-Public Network (SNPN) | IESNPN | S1 | SP-191038 | Wong, Curt; |
| 870023 | Management of non-public networks (NPN) | OAM_NPN | S5 | SP-200189 | ZHANG, Kai, Huawei |
| 860016 | Assurance Specification for IMS | SCAS_IMS | S3 | SP-191128 | Bo Zhang, Huawei Technologies |
| 870020 | Security Assurance Specification for 5G (eSCAS_5G) | eSCAS_5G | S3 | SP-200149 | Rong Wu, Huawei Technologies Co. |
| 890013 | eSCAS_5G for Network Slice-Specific Authentication and Authorization Function (NSSAAF) | SCAS_5G_NSSAAF | S3 | SP-200720 | Rong Wu, Huawei Technologies Co., Ltd. |
| 870017 | eSCAS_5G for Non-3GPP InterWorking Function | SCAS_5G_N3IWF | S3 | SP-200146 | Feng Gao, China Unicom |
| 870018 | eSCAS_5G for 5G NWDAF | SCAS_5G_NWDAF | S3 | SP-200147 | QI Minpeng, China Mobile |
| 870019 | eSCAS_5G for Service Communication Proxy | SCAS_5G_SECOP | S3 | SP-200148 | Wei Lu, Nokia |
| 880003 | eSCAS_5Gfor Inter PLMN UP Security | SCAS_5G_IPUPS | S3 | SP-200348 | Jin PENG, ZTE Corporation |
| 870022 | Enhancements on EE for 5G networks | EE5GPLUS | S5 | SP-200188 | Jean-Michel CORNILY, ORANGE |
| 870024 | Enhancement on Management Aspects of 5G Service-Level Agreement | EMA5SLA | S5 | SP-210860 | SHI, Xiaonan, CMCC |
| 870025 | Management of MDT enhancement in 5G | e_5GMDT | S5 | SP-200191 | Ayani, Zhulia, Ericsson |
| 870026 | Additional NRM features | adNRM | S5 | SP-200192 | Jing Ping, Nokia |
| 870027 | Enhancement of QoE Measurement Collection | eQoE | S5 | SP-200193 | Petersen, Robert, Ericsson |
| 860061 | Study on NR QoE (Quality of Experience) management and optimizations for diverse services | FS_NR_QoE | R3 | RP-193256 | China Unicom |
| 870030 | Enhanced Closed loop SLS Assurance | eCOSLA | S5 | SP-200196 | Groenendijk, Jan, Ericsson |
| 870034 | Enhanced Mission Critical Push-to-talk architecture phase 3 | enh3MCPTT |  | SP-200108 | Dom Lazara; Motorola Solutions |
| 870016 | Stage 2 aspects of enh3MCPTT | enh3MCPTT | S6 | SP-200108 | Dom Lazara; Motorola Solutions |
| 900039 | Stage 3 aspects of enh3MCPTT | enh3MCPTT-CT | C1 | CP-202188 |  |
| 840028 | IMS Charging in 5G System Architecture | 5GSIMSCH | S5 | SP-190367 | Jahangir, Zeeshan, T-Mobile USA |
| 810027 | Intent driven management service for mobile network | IDMS_MN | S5 | SP-180899 | Zou Lan, Huawei |
| 820006 | Study on User Plane Integrity Protection | FS_UP_IP_Sec | S3 | SP-181035 | Evans, Tim, Vodafone |
| 890012 | Enhancements to User Plane Integrity Protection Support in 5GS | eUPIP_SEC | S3 | SP-200719 | Anand Palanigounder, Qualcomm |
| 820035 | Discovery of management services in 5G | 5GDMS | S5 | SP-200770 | Brendan Hassett, Huawei Technologies Sweden AB |
| 860024 | Network policy managementfor 5G mobile networks | NPM | S5 | SP-191211 | China Mobile Shasha Guo |
| 860025 | Lawful Interception Rel-17 | LI17 | S3 | SP-190983 | Alex Leadbeater, BT |
| 850047 | (Small) Technical Enhancements and Improvements for Rel-17 | TEI17 |  |  |  |
| 870003 | IMS Optimization for HSS Group ID in an SBA environment | TEI17_IMSGID | S2 | SP-200452 | Foti, George, Ericsson |
| 910044 | Same PCF Selection For AMF and SMF | TEI17_SPSFAS |  | SP-200447 | Heng, Nie, China Telecom |
| 880016 | Stage 2 for TEI17_SPSFAS | TEI17_SPSFAS | S2 | SP-200447 | Heng, Nie, China Telecom |
| 910003 | CT aspects on TEI17_SPSFAS | TEI17_SPSFAS | ct | CP-211184 | Yue Sun, China Telecom |
| 910045 | CT3 aspects on TEI17_SPSFAS | TEI17_SPSFAS | C3 | CP-211184 | Yue Sun, China Telecom |
| 910046 | CT4 aspects on TEI17_SPSFAS | TEI17_SPSFAS | C4 | CP-211184 | Yue Sun, China Telecom |
| 930036 | System enhancement for Redundant PDU Session | TEI17_SE_RPS |  | SP-200448 | Devaki Chandramouli |
| 880017 | Stage 2 for TEI17_SE_RPS | TEI17_SE_RPS | S2 | SP-200448 | Devaki Chandramouli |
| 930001 | CT aspects of TEI17_SE_RPS | TEI17_SE_RPS | ct | CP-212099 | WON, Sung Hwan, Nokia |
| 930037 | CT1 aspects of TEI17_SE_RPS | TEI17_SE_RPS | C1 | CP-212099 | WON, Sung Hwan, Nokia |
| 930038 | CT3 aspects of TEI17_SE_RPS | TEI17_SE_RPS | C3 | CP-212099 | WON, Sung Hwan, Nokia |
| 930039 | CT4 aspects of TEI17_SE_RPS | TEI17_SE_RPS | C4 | CP-212099 | WON, Sung Hwan, Nokia |
| 910078 | Support for Signed Attestation for Priority and Emergency Sessions | TEI17_SAPES |  | SP-200453 | Foti, George, Ericsson |
| 870004 | Stage 2 for TEI17_SAPES | TEI17_SAPES | S2 | SP-200453 | Foti, George, Ericsson |
| 910020 | CT aspects on TEI17_SAPES | TEI17_SAPES | ct | CP-210272 | Axell, Jörgen, Ericsson |
| 910079 | CT1 aspects on TEI17_SAPES | TEI17_SAPES | C1 | CP-210272 | Axell, Jörgen, Ericsson |
| 910080 | CT3 aspects on TEI17_SAPES | TEI17_SAPES | C3 | CP-210272 | Axell, Jörgen, Ericsson |
| 910081 | Dynamically Changing AM Policies in the 5GC | TEI17_DCAMP |  | SP-200446 | Pancorbo, Belen (Ericsson) |
| 870005 | Stage 2 for TEI17_DCAMP | TEI17_DCAMP | S2 | SP-200446 | Pancorbo, Belen (Ericsson) |
| 910009 | CT aspects on TEI17_DCAMP | TEI17_DCAMP | ct | CP-212163 | Garcia Azorero, Fuencisla. Ericsson |
| 910082 | CT3 aspects on TEI17_DCAMP | TEI17_DCAMP | C3 | CP-212163 | Garcia Azorero, Fuencisla. Ericsson |
| 910083 | CT4 aspects on TEI17_DCAMP | TEI17_DCAMP | C4 | CP-212163 | Garcia Azorero, Fuencisla. Ericsson |
| 870006 | IP address pool information from UDM | TEI17_IPU | S2 | SP-200454 | Stefan Rommer |
| 910084 | Dynamic management of group-based event monitoring | TEI17_GEM |  | SP-200455 | Qian XB Chen, Ericsson |
| 870007 | Stage 2 for TEI17_GEM | TEI17_GEM | S2 | SP-200455 | Qian XB Chen, Ericsson |
| 910010 | CT aspects on TEI17_GEM | TEI17_GEM | ct | CP-212165 | Maria Tianmei, Liang, Ericsson |
| 910085 | CT3 aspects on TEI17_GEM | TEI17_GEM | C3 | CP-212165 | Maria Tianmei, Liang, Ericsson |
| 910086 | CT4 aspects on TEI17_GEM | TEI17_GEM | C4 | CP-212165 | Maria Tianmei, Liang, Ericsson |
| 920041 | Support of different slices over different Non 3GPP access | TEI17_N3SLICE |  | SP-200456 | Laurent Thiebaut, Nokia |
| 870008 | Stage 2 for TEI17_N3SLICE | TEI17_N3SLICE | S2 | SP-200456 | Laurent Thiebaut, Nokia |
| 930099 | CT1 aspects of TEI17_N3SLICE | TEI17_N3SLICE | C1 | CP-211089 | LANDAIS, Bruno, Nokia |
| 920001 | CT4 aspects of TEI17_N3SLICE | TEI17_N3SLICE | C4 | CP-211089 | LANDAIS, Bruno, Nokia |
| 910087 | N7/N40 Interfaces Enhancements to Support GERAN and UTRAN | TEI17_NIESGU |  | SP-200457 | Aihua, Li, China Mobile |
| 870009 | Stage 2 for TEI17_NIESGU | TEI17_NIESGU | S2 | SP-200457 | Aihua, Li, China Mobile |
| 890019 | SA5 aspects of N40 Interface Enhancements to Support GERAN and UTRAN | TEI17_NIESGU | S5 | SP-200854 | Dong, Jia, China Mobile |
| 910011 | CT3 aspects of N7 Interfaces Enhancements to Support GERAN and UTRAN | TEI17_NIESGU | C3 | CP-211194 | Huang Zhenning (China Mobile) |
| 920082 | CT4 aspects of N7 Interfaces Enhancements to Support GERAN and UTRAN | TEI17_NIESGU | C4 | CP-211194 | Huang Zhenning (China Mobile) |
| 880019 | Stage-3 5GS NAS protocol development 17 | 5GProtoc17 | C1 | CP-201163 | Sedlacek, Ivo, Ericsson |
| 880044 | Stage-3 5GS NAS protocol development 17 general aspects | 5GProtoc17 | C1 | CP-201163 | Sedlacek, Ivo, Ericsson |
| 880045 | Stage-3 5GS NAS protocol development 17 non-IETF aspects | 5GProtoc17-non3GPP | C1 | CP-201163 | Sedlacek, Ivo, Ericsson |
| 880021 | Stage-3 SAE Protocol Development | SAES17 | C1 | CP-201165 | Aghili, Behrouz, InterDigital Communications |
| 880046 | Stage-3 SAE Protocol Development general aspects | SAES17 | C1 | CP-201165 | Aghili, Behrouz, InterDigital Communications |
| 880047 | Stage-3 SAE Protocol Development CSFB aspects | SAES17-CSFB | C1 | CP-201165 | Aghili, Behrouz, InterDigital Communications |
| 880048 | Stage-3 SAE Protocol Development non 3GPP aspects | SAES17-non3GPP | C1 | CP-201165 | Aghili, Behrouz, InterDigital Communications |
| 911034 | IMS Stage-3 IETF Protocol Alignment | IMSProtoc17 | ct | CP-201167 | Gkatzikis, Lazaros(Nokia) |
| 880023 | CT1 aspects of IMS Stage-3 IETF Protocol Alignment | IMSProtoc17 | C1 | CP-201167 | Gkatzikis, Lazaros(Nokia) |
| 911035 | CT3 aspects of IMS Stage-3 IETF Protocol Alignment | IMSProtoc17 | C3 | CP-201167 | Gkatzikis, Lazaros(Nokia) |
| 880032 | Packet Flow Description (PFD) management enhancement | pfdManEnh | C3 | CP-210183 | Xiaoyun Zhou, Huawei |
| 880013 | Service Based Interface Protocol Improvements Release 17 | SBIProtoc17 |  | CP-211088 | Song Yue, China Mobile |
| 880053 | CT3 aspects of SBIProtoc17 | SBIProtoc17 | C3 | CP-211088 | Song Yue, China Mobile |
| 880054 | CT4 aspects of SBIProtoc17 | SBIProtoc17 | C4 | CP-211088 | Song Yue, China Mobile |
| 850028 | Study on enhancement of Management Data Analytics Service | FS_eMDAS | S5 | SP-190930 | Yao, Yizhi, Intel |
| 910027 | Enhancements of Management Data Analytics Service | eMDAS | S5 | SP-210132 | Yao, Yizhi, Intel |

Summary based on the input provided by XXX in xP-21xxxx.

Annex A:
List of expected contributions

Summary needed for highlighted items. Summary to be produced by the Rapporteur as highlighted in the table below.

| UID | Name | A c r o n y m | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 0 | Rel-17 on Use of 5G by different sectors ("verticals") | - |  | - | - |
| 840050 | Enhancements for cyber-physical control applications in vertical domains | e C A V |  | SP-190310 | Bahr, Michael, Siemens AG |
| 830020 | Study on eCAV | F S _ e C A V | S1 | SP-190092 | Michael Bahr, Siemens AG |
| 840041 | Stage 1 of eCAV | e C A V | S1 | SP-191043 | Bahr, Michael, Siemens AG |
| 840024 | Study on enhanced support of Non-Public Networks | F S _ e N P N | S2 | SP-200094 | Peter Hedman |
| 910065 | Enhanced support of Non-Public Networks | e N P N |  | SP-200980 | Hedman, Peter, Ericsson |
| 900015 | Stage 2 for eNPN | e N P N | S2 | SP-200980 | Hedman, Peter, Ericsson |
| 910016 | CT aspects of eNPN | e N P N | ct | CP-212103 | Sedlacek, Ivo, Ericsson |
| 910066 | CT1 aspects of eNPN | e N P N | C1 | CP-212103 | Sedlacek, Ivo, Ericsson |
| 910067 | CT3 aspects of eNPN | e N P N | C3 | CP-212103 | Sedlacek, Ivo, Ericsson |
| 910068 | CT4 aspects of eNPN | e N P N | C4 | CP-212103 | Sedlacek, Ivo, Ericsson |
| 920025 | Security Aspects of eNPN | e N P N | S3 | SP-210422 | Jost, Christine, Ericsson |
| 880008 | Study on enhanced security support for Non-Public Networks | F S _ e N P N _ S E C | S3 | SP-200353 | Normann, Henrik, Ericsson |
| 840045 | Audio-Visual Service Production | A V P R O D |  | SP-190304 | Wagdin, Ian, BBC |
| 800014 | Study on AVPROD | F S _ A V P R O D | S1 | SP-181015 | Roland Beutler, EBU |
| 840031 | Stage 1 of AVPROD | A V P R O D | S1 | SP-191041 | Wagdin, Ian, BBC |
| 910001 | Study on Media Production over 5G NPN | F S _ N P N 4 A V P r o d | S4 | SP-210241 | Lohmar, Thorsten, Ericsson |
| 820025 | Study on application layer support for Factories of the Future in 5G network | F S _ F F A P P | S6 | SP-200836 | Shao Weixiang, ZTE Corporation |
| 840025 | Study on enhancement of support for 5G LAN-type service | F S _ 5 G L A N _ e n h | S2 | SP-190626 | Runze Zhou, Huawei |
| 900022 | Study on Charging Aspect of 5G LAN-type Services | F S _ 5 G L A N _ C H | S5 | SP-201081 | CHEN SHAN, Huawei |
| 840042 | Complete Gap Analysis for Railways Mobile Communication System | M O N A S T E R Y E N D | S1 | SP-190312 | Gach, Guillaume, UIC |
| 890035 | Enhancements to Application Architecture for the Mobile Communication System for Railways Phase 2 | e M O N A S T E R Y 2 |  | SP-191104 | Oettl, Martin, Nokia |
| 840037 | Stage 2 of eMONASTERY2 | e M O N A S T E R Y 2 | S6 | SP-191104 | Oettl, Martin, Nokia |
| 890063 | Stage 3 of eMONASTERY2 | e M O N A S T E R Y 2 | C1 | CP-202256 | Gkatzikis, Lazaros, Nokia(Nokia) |
| 840043 | Study on Future Railway Mobile Communication System3 | F S _ F R M C S 3 | S1 | SP-200578 | Gach, Guillaume, UIC, |
| 850046 | Asset Tracking for 5G | A T R A C |  | SP-190816 | Thierry Berisot, NOVAMINT |
| 810017 | Study on ATRAC | F S _ 5 G _ A T R A C | S1 | SP-180922 | Thierry Berisot, NOVAMINT |
| 850037 | Stage 1 of ATRAC | A T R A C | S1 | SP-190931 | Thierry Berisot, NOVAMINT |
| 910052 | Enhancement to the 5GC LoCation Services-Phase 2 | 5 G _ e L C S _ p h 2 |  | SP-200082 | Ming Ai, CATT |
| 870001 | Stage 2 of 5G_eLCS_ph2 | 5 G _ e L C S _ p h 2 | S2 | SP-200082 | Ming Ai, CATT |
| 910006 | CT aspects of 5G_eLCS_ph2 | 5 G _ e L C S _ p h 2 | ct | CP-211090 | Chenxi Bao, CATT |
| 910053 | CT1 aspects of 5G_eLCS_ph2 | 5 G _ e L C S _ p h 2 | C1 | CP-211090 | Chenxi Bao, CATT |
| 910054 | CT3 aspects of 5G_eLCS_ph2 | 5 G _ e L C S _ p h 2 | C3 | CP-211090 | Chenxi Bao, CATT |
| 910055 | CT4 aspects of 5G_eLCS_ph2 | 5 G _ e L C S _ p h 2 | C4 | CP-211090 | Chenxi Bao, CATT |
| 900060 | RAN aspects of NR positioning enhancements | N R _ p o s _ e n h |  | RP-210903 | Intel |
| 860034 | Study on NR positioning enhancements | F S _ N R _ p o s _ e n h | R1 | RP-202094 | CATT |
| 900160 | Core part: NR positioning enhancements | N R _ p o s _ e n h - C o r e | R1 | RP-210903 | Intel |
| 900260 | Perf. part: NR positioning enhancements | N R _ p o s _ e n h - P e r f | R4 | RP-210903 | Intel |
| 840047 | Communication Service Requirements for Critical Medical Applications | C M E D |  | SP-190306 | Lagrange, Mathieu, b<>com |
| 810016 | Study on CMED | F S _ C M E D | S1 | SP-180783 | Mathieu Lagrange, b<>com |
| 840033 | Stage 1 of CMED | C M E D | S1 | SP-190306 | Lagrange, Mathieu, b<>com |
| 0 | Rel-17 drone/UAS/UAV/EAV | - |  | - | - |
| 840083 | 5G Enhancement for UAVs | E A V |  | SP-190308 | Qun Wei, China Unicom |
| 810019 | Study on EAV | F S _ E A V | S1 | SP-180909 | Qun Wei, China Unicom |
| 840039 | Stage 1 of EAV | E A V | S1 | SP-190308 | Qun Wei, China Unicom |
| 820011 | Study on supporting Uncrewed Aerial Systems Connectivity, Identification, and Tracking | F S _ I D _ U A S _ S A 2 | S2 | SP-200097 | Qualcomm, Stefano Faccin |
| 820026 | Study on application layer support for Uncrewed Aerial System (UAS) | F S _ U A S A P P | S6 | SP-200111 | Niranth Amogh, Huawei |
| 920045 | Application layer support for Uncrewed Aerial System (UAS) | U A S A P P |  | SP-200988 | Niranth Amogh, Huawei Telecommunications India |
| 900025 | Stage 2 of Application layer support for Uncrewed Aerial System (UAS) | U A S A P P | S6 | SP-200988 | Niranth Amogh, Huawei Telecommunications India |
| 920004 | CT1 Aspects of Application Layer Support for Uncrewed Aerial Systems (UAS) | U A S A P P | C1 | CP-211111 | Lin Shu, Huawei, |
| 920046 | CT3 Aspects of Application Layer Support for Uncrewed Aerial Systems (UAS) | U A S A P P | C3 | CP-211111 | Lin Shu, Huawei |
| 880007 | Study on security aspects of Uncrewed Aerial Systems (UAS) | F S _ U A S _ S E C | S3 | SP-200352 | Adrian Escott, Qualcomm |
| 810049 | Remote Identification of Uncrewed Aerial Systems | I D _ U A S |  | SP-180771 | Eddy HALL, Qualcomm |
| 810013 | Stage 1 of Remote Identification of Uncrewed Aerial Systems | I D _ U A S | S1 | SP-180771 | Eddy HALL, Qualcomm |
| 900014 | (Stage 2 of) Support of Uncrewed Aerial Systems Connectivity, Identification, and Tracking | I D _ U A S | S2 | SP-200979 | Faccin, Stefano, Qualcomm |
| 910017 | CT aspects for ID_UAS | I D _ U A S | ct | CP-211112 | Sunghoon Kim, Qualcomm |
| 910069 | CT1 aspects for ID_UAS | I D _ U A S | C1 | CP-211112 | Sunghoon Kim, Qualcomm |
| 910070 | CT3 aspects for ID_UAS | I D _ U A S | C3 | CP-211112 | Sunghoon Kim, Qualcomm |
| 910071 | CT4 aspects for ID_UAS | I D _ U A S | C4 | CP-211112 | Sunghoon Kim, Qualcomm |
| 920028 | Security aspects of Uncrewed Aerial Systems | I D _ U A S | S3 | SP-210425 | Escott, Adrian, Qualcomm |
| 0 | Rel-17 Satellite/Non-Terrestrial (NTN) related | - |  | - | - |
| 890034 | Integration of satellite components in the 5G architecture | 5 G S A T _ A R C H |  | SP-191335 | Cyril MICHEL, thales |
| 800048 | Stage 1 of 5GSAT | 5 G S A T | S1 | SP-180326 | MICHEL, Cyril, Thales |
| 800026 | Study on architecture aspects for using satellite access in 5G | F S _ 5 G S A T _ A R C H | S2 | SP-181253 | MICHEL, Cyril |
| 860005 | (Stage 2 of) Integration of satellite components in the 5G architecture | 5 G S A T _ A R C H | S2 | SP-191335 | Cyril MICHEL, thales |
| 911030 | CT aspects of 5GC architecture for satellite networks | 5 G S A T _ A R C H - C T | ct | CP-210149 | Catovic, Amer, Qualcomm |
| 890005 | CT1 aspects of 5GC architecture for satellite networks | 5 G S A T _ A R C H - C T | C1 | CP-210149 | Catovic, Amer, Qualcomm |
| 920057 | CT3 aspects of 5GC architecture for satellite networks | 5 G S A T _ A R C H - C T | C3 | CP-210149 | Catovic, Amer, Qualcomm |
| 911031 | CT4 aspects of 5GC architecture for satellite networks | 5 G S A T _ A R C H - C T | C4 | CP-210149 | Catovic, Amer, Qualcomm |
| 930044 | CT6 aspects of 5GC architecture for satellite networks | 5 G S A T _ A R C H - C T | C6 | CP-210149 | Catovic, Amer, Qualcomm |
| 860046 | Solutions for NR to support non-terrestrial networks (NTN) | N R _ N T N _ s o l u t i o n s |  | RP-210908 | Thales |
| 860146 | Core part: Solutions for NR for NTN | N R _ N T N _ s o l u t i o n s - C o r e | R2 | RP-210908 | Thales |
| 860246 | Perf. Part: Solutions for NR for NTN | N R _ N T N _ s o l u t i o n s - P e r f | R4 | RP-210908 | Thales |
| 920069 | NB-IoT/eMTC support for Non-Terrestrial Networks | L T E _ N B I O T _ e M T C _ N T N | R1 | RP-211601 | MediaTek |
| 920169 | Core part: NB-IoT/eMTC support for Non-Terrestrial Networks | L T E _ N B I O T _ e M T C _ N T N - C o r e | R1 | RP-211601 | MediaTek |
| 930019 | Architecture support for NB-IoT/eMTC Non-Terrestrial Networks in EPS | I o T _ S A T _ A R C H _ E P S | S2 | SP-211124 | SEBIRE, Guillaume, MediaTek Inc. |
| 830025 | Study on management and orchestration aspects with integrated satellite components in a 5G network | F S _ 5 G S A T _ M O | S5 | SP-190138 | Floris Drijver, TNO |
| 860033 | Study on NB-IoT/eMTC support for NTN | F S _ L T E _ N B I O T _ e M T C _ N T N | R1 | RP-202689 | MediaTek |
| 0 | Rel-17 IoT related | - |  | - | - |
| 850012 | Study on enhanced support of Industrial IoT | F S _ I I o T | S2 | SP-200298 | Nokia, Devaki Chandramouli, |
| 880010 | Study on security for enhanced support of Industrial IoT | F S _ I I o T _ S E C | S3 | SP-200355 | Nokia, Anja Jerichow |
| 920024 | Security for enhanced support of Industrial IoT | I I o T _ S E C | S3 | SP-210421 | Nokia, Anja Jerichow |
| 910059 | Support of Enhanced Industrial IIoT | I I o T |  | SP-200973 | Devaki Chandramouli, Nokia |
| 900008 | Stage 2 for IIoT | I I o T | S2 | SP-200973 | Devaki Chandramouli, Nokia |
| 910014 | CT aspects of support of enhanced Industrial IoT | I I o T | ct | CP-212100 | Won, Sung Hwan, Nokia |
| 910060 | CT1 aspects of support of enhanced Industrial IoT | I I o T | C1 | CP-212100 | Won, Sung Hwan, Nokia |
| 910061 | CT3 aspects of support of enhanced Industrial IoT | I I o T | C3 | CP-212100 | Won, Sung Hwan, Nokia |
| 910062 | CT4 aspects of support of enhanced Industrial IoT | I I o T | C4 | CP-212100 | Won, Sung Hwan, Nokia |
| 860044 | Additional enhancements for NB-IoT and LTE-MTC | N B _ I O T e n h 4 _ L T E _ e M T C 6 |  | RP-211340 | Huawei |
| 860144 | Core part: Additional enhancements for NB-IoT and LTE-MTC | N B _ I O T e n h 4 _ L T E _ e M T C 6 - C o r e | R1 | RP-211340 | Huawei |
| 860244 | Perf. part: Additional enhancements for NB-IoT and LTE-MTC | N B _ I O T e n h 4 _ L T E _ e M T C 6 - P e r f | R4 | RP-211340 | Huawei |
| 860051 | NR small data transmissions in INACTIVE state | N R _ S m a l l D a t a _ I N A C T I V E |  | RP-212594 | ZTE |
| 860151 | Core part: NR small data transmissions in INACTIVE state | N R _ S m a l l D a t a _ I N A C T I V E - C o r e | R2 | RP-212594 | ZTE |
| 860251 | Perf. part: NR small data transmissions in INACTIVE state | N R _ S m a l l D a t a _ I N A C T I V E - P e r f | R4 | RP-212594 | ZTE |
| 860045 | Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR | N R _ I I O T _ U R L L C _ e n h |  | RP-210854 | Nokia |
| 860145 | Core part: Enhanced IoT and URLLC support for NR | N R _ I I O T _ U R L L C _ e n h - C o r e | R2 | RP-210854 | Nokia |
| 860245 | Perf. Part: Enhanced IoT and URLLC support for NR | N R _ I I O T _ U R L L C _ e n h - P e r f | R4 | RP-210854 | Nokia |
| 0 | Rel-17 V2V/D2D/Sidelink related | - |  | - | - |
| 860042 | NR Sidelink enhancement | N R _ S L _ e n h |  | RP-202846 | LG Electronics |
| 860142 | Core part: NR Sidelink enhancement | N R _ S L _ e n h - C o r e | R1 | RP-202846 | LG Electronics |
| 860242 | Perf. part: NR Sidelink enhancement | N R _ S L _ e n h - P e r f | R4 | RP-202846 | LG Electronics |
| 860038 | Study on NR Sidelink relay | F S _ N R _ S L _ r e l a y | R2 | RP-202208 | OPPO |
| 840035 | Study on enhancements to application layer support for V2X services | F S _ e V 2 X A P P | S6 | SP-200110 | Niranth Amogh, Huawei Telecommunications India |
| 910075 | Enhanced application layer support for V2X services | e V 2 X A P P |  | SP-200831 | Niranth Amogh, Huawei Telecommunications India |
| 890036 | Stage 2 of eV2XAPP | e V 2 X A P P | S6 | SP-200831 | Niranth Amogh, Huawei Telecommunications India |
| 910019 | CT aspects of eV2XAPP | e V 2 X A P P | ct | CP-211109 | Herrero Veron, Christian (Huawei) |
| 910076 | CT1 aspects of eV2XAPP | e V 2 X A P P | C1 | CP-211109 | Herrero Veron, Christian (Huawei) |
| 910077 | CT3 aspects of eV2XAPP | e V 2 X A P P | C3 | CP-211109 | Herrero Veron, Christian (Huawei) |
| 910037 | Support of advanced V2X services - Phase 2 | e V 2 X A R C _ P h 2 |  | SP-210090 | LaeYoung Kim, LG Electronics |
| 850013 | Study on V2X services – Phase 2 | F S _ e V 2 X A R C _ P h 2 | S2 | SP-190631 | LaeYoung Kim, LG Electronics |
| 910021 | Stage 2 of eV2XARC_Ph2 | e V 2 X A R C _ P h 2 | S2 | SP-210090 | LaeYoung Kim, LG Electronics |
| 920005 | CT1 aspects of eV2XARC_Ph2 | e V 2 X A R C _ P h 2 | C1 | CP-211116 | Herrero Veron, Christian (Huawei) |
| 920047 | CT3 aspects of eV2XARC_Ph2 | e V 2 X A R C _ P h 2 | C3 | CP-211116 | Herrero Veron, Christian (Huawei) |
| 920048 | CT6 aspects of eV2XARC_Ph2 | e V 2 X A R C _ P h 2 | C6 | CP-211116 | Herrero Veron, Christian (Huawei) |
| 0 | Rel-17 NR enhancements | - |  | - | - |
| 920070 | UE RF requirements for Transparent Tx Diversity (TxD) for NR | N R _ R F _ T x D | R4 | RP-211940 | Qualcomm |
| 920170 | Core part: UE RF requirements for Transparent Tx Diversity (TxD) for NR | N R _ R F _ T x D - C o r e | R4 | RP-211940 | Qualcomm |
| 920270 | Perf. part: UE RF requirements for Transparent Tx Diversity (TxD) for NR | N R _ R F _ T x D - P e r f | R4 | RP-211940 | Qualcomm |
| 860040 | Further enhancements on MIMO for NR | N R _ f e M I M O |  | RP-212535 | Samsung |
| 860140 | Core part: Further enhancements on MIMO for NR | N R _ f e M I M O - C o r e | R1 | RP-212535 | Samsung |
| 860240 | Perf. part: Further enhancements on MIMO for NR | N R _ f e M I M O - P e r f | R4 | RP-212535 | Samsung |
| 860043 | NR Dynamic spectrum sharing (DSS) | N R _ D S S |  | RP-211345 | Ericsson |
| 860143 | Core part: NR DSS | N R _ D S S - C o r e | R1 | RP-211345 | Ericsson |
| 860047 | UE power saving enhancements for NR | N R _ U E _ p o w _ s a v _ e n h |  | RP-212630 | MediaTek |
| 860147 | Core part: UE power saving enhancements for NR | N R _ U E _ p o w _ s a v _ e n h - C o r e | R2 | RP-212630 | MediaTek |
| 860247 | Perf. part: UE power saving enhancements for NR | N R _ U E _ p o w _ s a v _ e n h - P e r f | R4 | RP-212630 | MediaTek |
| 860049 | Further Multi-RAT Dual-Connectivity enhancements | L T E _ N R _ D C _ e n h 2 |  | RP-201040 | Huawei |
| 860149 | Core part: Further Multi-RAT Dual-Connectivity enhancements | L T E _ N R _ D C _ e n h 2 - C o r e | R2 | RP-201040 | Huawei |
| 860249 | Perf. part: Further Multi-RAT Dual-Connectivity enhancements | L T E _ N R _ D C _ e n h 2 - P e r f | R4 | RP-201040 | Huawei |
| 860050 | Enhancements to Integrated Access and Backhaul (IAB) for NR | N R _ I A B _ e n h |  | RP-211548 | Qualcomm |
| 860150 | Core part: Enhancements to IAB for NR | N R _ I A B _ e n h - C o r e | R2 | RP-211548 | Qualcomm |
| 860250 | Perf. part: Enhancements to IAB for NR | N R _ I A B _ e n h - P e r f | R4 | RP-211548 | Qualcomm |
| 830021 | Study on Security for NR_IAB | F S _ N R _ I A B _ S e c | S3 | SP-201016 | Rajavelsamy Rajadurai, Samsung, |
| 860052 | Enhanced eNB(s) architecture evolution for E-UTRAN and NG-RAN | L T E _ N R _ a r c h _ e v o _ e n h |  | RP-211409 | China Unicom |
| 860152 | Core part: LTE_NR_arch_evo_enh | L T E _ N R _ a r c h _ e v o _ e n h - C o r e | R3 | RP-211409 | China Unicom |
| 890055 | Further enhancement on NR demodulation performance | N R _ d e m o d _ e n h 2 |  | RP-212636 | China Telecom |
| 890255 | Perf. part: NR_demod_enh2 | N R _ d e m o d _ e n h 2 - P e r f | R4 | RP-212636 | China Telecom |
| 890056 | Introduction of DL 1024QAM for NR frequency range 1 (FR1) | N R _ D L 1 0 2 4 Q A M _ F R 1 |  | RP-202886 | Ericsson |
| 890156 | Core part: Introduction of DL 1024QAM for NR FR1 | N R _ D L 1 0 2 4 Q A M _ F R 1 - C o r e | R4 | RP-202886 | Ericsson |
| 890256 | Perf. part: Introduction of DL 1024QAM for NR FR1 | N R _ D L 1 0 2 4 Q A M _ F R 1 - P e r f | R4 | RP-202886 | Ericsson |
| 890057 | NR RRM further enhancement | N R _ R R M _ e n h 2 |  | RP-202874 | Apple |
| 890157 | Core part: Further RRM enhancement for NR and MR-DC | N R _ R R M _ e n h 2 - C o r e | R4 | RP-202874 | Apple |
| 890257 | Perf. part: Further RRM enhancement for NR and MR-DC | N R _ R R M _ e n h 2 - P e r f | R4 | RP-202874 | Apple |
| 890058 | Enhanced NR support for high speed train scenario for frequency range 1 (FR1) | N R _ H S T _ F R 1 _ e n h |  | RP-210833 | CMCC |
| 890158 | Core part: Enhanced NR support for high speed train scenario for FR1 | N R _ H S T _ F R 1 _ e n h - C o r e | R4 | RP-210833 | CMCC |
| 890258 | Perf. part: Enhanced NR support for high speed train scenario for FR1 | N R _ H S T _ F R 1 _ e n h - P e r f | R4 | RP-210833 | CMCC |
| 890059 | Further enhancements of NR RF requirements for frequency range 2 (FR2) | N R _ R F _ F R 2 _ r e q _ e n h 2 |  | RP-212092 | Nokia |
| 890159 | Core part: NR_RF_FR2_req_enh2 | N R _ R F _ F R 2 _ r e q _ e n h 2 - C o r e | R4 | RP-212092 | Nokia |
| 890259 | Perf. part: NR_RF_FR2_req_enh2 | N R _ R F _ F R 2 _ r e q _ e n h 2 - P e r f | R4 | RP-212092 | Nokia |
| 890060 | NR support for high speed train scenario in frequency range 2 (FR2) | N R _ H S T _ F R 2 |  | RP-210800 | Samsung |
| 890160 | Core part: NR support for high speed train scenario in FR2 | N R _ H S T _ F R 2 - C o r e | R4 | RP-210800 | Samsung |
| 890260 | Perf. Part: NR support for high speed train scenario in FR2 | N R _ H S T _ F R 2 - P e r f | R4 | RP-210800 | Samsung |
| 890061 | NR measurement gap enhancements | N R _ M G _ e n h |  | RP-211591 | MediaTek |
| 890161 | Core part: NR and MR-DC measurement gap enhancements | N R _ M G _ e n h - C o r e | R4 | RP-211591 | MediaTek |
| 890261 | Perf. part: NR and MR-DC measurement gap enhancements | N R _ M G _ e n h - P e r f | R4 | RP-211591 | MediaTek |
| 890062 | RF requirements enhancement for NR frequency range 1 (FR1) | N R _ R F _ F R 1 _ e n h |  | RP-212527 | Huawei |
| 890162 | Core part: RF requirements enhancement for NR FR1 | N R _ R F _ F R 1 _ e n h - C o r e | R4 | RP-212527 | Huawei |
| 890262 | Perf. part: RF requirements enhancement for NR FR1 | N R _ R F _ F R 1 _ e n h - P e r f | R4 | RP-212527 | Huawei |
| 0 | Rel-17 Self-Organizing (SON)/Autonomous Networks | - |  | - | - |
| 860053 | Enhancement of data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR and EN-DC | N R _ E N D C _ S O N _ M D T _ e n h |  | RP-193255 | CMCC |
| 860153 | Core part: Enhancement of data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR and EN-DC | N R _ E N D C _ S O N _ M D T _ e n h - C o r e | R3 | RP-193255 | CMCC |
| 910029 | Plug and connect support for management of Network Functions | P A C M A N | S5 | SP-210260 | Per Elmdahl, Ericsson |
| 870028 | Enhancements of Self-Organizing Networks (SON) for 5G networks | e S O N _ 5 G | S5 | SP-200194 | Joey Chou, Intel |
| 850032 | Study on autonomous network levels | F S _ A N L | S5 | SP-190928 | China Mobile |
| 880027 | Autonomous network levels | A N L | S5 | SP-200464 | Cao, Xi, China Mobile |
| 0 | Rel-17 Multicast-broadcast Services (MBS) and Mission Critical (MC) | - |  | - | - |
| 900038 | Multicast-broadcast services in 5G | 5 M B S |  | SP-201106 | Meng Li, Huawei |
| 830030 | Study on Architectural enhancements for 5G multicast-broadcast services | F S _ 5 M B S | S2 | SP-200690 | Meng Li, Huawei |
| 900009 | Stage 2 for 5MBS | 5 M B S | S2 | SP-201106 | Meng Li, Huawei |
| 920043 | CT1 aspects of the architectural enhancements for 5G multicast-broadcast services | 5 M B S | C1 | CP-212022 | Gulbani, Giorgi, Huawei |
| 920044 | CT3 aspects of the architectural enhancements for 5G multicast-broadcast services | 5 M B S | C3 | CP-212022 | Gulbani, Giorgi, Huawei |
| 910002 | CT4 aspects of the architectural enhancements for 5G multicast-broadcast services | 5 M B S | C4 | CP-212022 | Gulbani, Giorgi, Huawei |
| 920023 | Security Aspects of Enhancements for 5G Multicast-Broadcast Services | 5 M B S | S3 | SP-210420 | Longhua Guo, Huawei Technologies |
| 870014 | Study on Multicast Architecture Enhancements for 5G Media Streaming | F S _ 5 G M S _ M u l t i c a s t | S4 | SP-200238 | Peng Tan, Telus |
| 810055 | Study on location enhancements for mission critical services | F S _ e n h M C L o c | S6 | SP-190725 | Dom Lazara, Motorola Solutions |
| 850040 | Broadcast / Multicast requirements supporting Mission Critical Services in 5G | 5 M B S _ e M C | S1 | SP-190942 | Toobe, Jens, BDBOS |
| 880006 | Study on Security Aspects of Enhancements for 5G Multicast-Broadcast Services | F S _ 5 M B S _ S E C | S3 | SP-200351 | Longhua Guo, Huawei Technologies |
| 850035 | Study on Mission Critical services over 5G multicast-broadcast system | F S _ M C 5 M B S | S6 | SP-190929 | Val Oprescu, AT&T |
| 860048 | NR multicast and broadcast services | N R _ M B S | R2 | RP-193248 | Huawei |
| 860148 | Core part: NR multicast and broadcast services | N R _ M B S - C o r e | R2 | RP-201038 | Huawei |
| 800023 | Study on Mission Critical services support over 5G System | F S _ M C O v e r 5 G S | S6 | SP-200837 | Verweij, Kees, The Police of the Netherlands |
| 920051 | Mission Critical Services over 5GS | M C O v e r 5 G S |  | SP-200833 | Wendler, Ingo, UIC |
| 890027 | Stage 2 of MCOver5GS | M C O v e r 5 G S | S6 | SP-200833 | Wendler, Ingo, UIC |
| 920007 | CT1 aspects of MCOver5GS | M C O v e r 5 G S | C1 | CP-211118 | Gkatzikis, Lazaros, Nokia |
| 890039 | Mission Critical Data | e M C D a t a 3 |  | SP-191106 | Shih, Jerry, AT&T |
| 860007 | Stage 2 of MCData3 | e M C D a t a 3 | S6 | SP-191106 | Shih, Jerry, AT&T |
| 890038 | CT aspects of eMCData3 | e M C D a t a 3 | C1 | CP-201177 |  |
| 880020 | Protocol enhancements for Mission Critical Services | M C P r o t o c 1 7 | C1 | CP-202193 | AXELL, Jörgen |
| 820040 | Mission Critical system migration and interconnection | M C S M I _ C T | C1 | CP-190143 | Dom Lazara, Motorola Solutions |
| 890003 | CT aspects of Enhanced Mission Critical Communication Interworking with Land Mobile Radio Systems | e M C C I _ C T | C1 | eMCCI_CT | Mike Dolan, FirstNet |
| 0 | Rel-17 Edge computing | - |  | - | - |
| 880042 | Architecture for enabling Edge Applications | E D G E A P P |  | SP-200109 | Gupta, Nishant, Samsung |
| 830008 | Study on Application Architecture for enabling Edge Applications | F S _ E D G E A P P | S6 | SP-190065 | Nishant Gupta, Samsung |
| 860006 | Architecture for enabling Edge Applications | E D G E A P P | S6 | SP-200886 | Gupta, Nishant, Samsung |
| 900006 | CT aspects for Enabling Edge Applications | E D G E A P P |  | CP-211196 | Narendranath Durga Tangudu |
| 900034 | CT1 aspects for Enabling Edge Applications | E D G E A P P | C1 | CP-211196 | Narendranath Durga Tangudu |
| 900035 | CT3 aspects for Enabling Edge Applications | E D G E A P P | C3 | CP-211196 | Narendranath Durga Tangudu |
| 830032 | Study on enhancement of support for Edge Computing in 5GC | F S _ e n h _ E C | S2 | SP-200093 | Hui Ni, Huawei |
| 910048 | Enhancement of support for Edge Computing in 5G Core network | e E D G E _ 5 G C |  | SP-201107 | Hui Ni, Huawei Technologies |
| 900016 | Stage 2 of eEDGE_5GC | e E D G E _ 5 G C | S2 | SP-201107 | Hui Ni, Huawei Technologies |
| 910005 | CT aspects of 5G eEDGE | e E D G E _ 5 G C | ct | CP-212021 | Qi Caixia, Huawei |
| 910049 | CT1 aspects of 5G eEDGE | e E D G E _ 5 G C | C1 | CP-212021 | Qi Caixia, Huawei |
| 910050 | CT3 aspects of 5G eEDGE | e E D G E _ 5 G C | C3 | CP-212021 | Qi Caixia, Huawei |
| 910051 | CT4 aspects of 5G eEDGE | e E D G E _ 5 G C | C4 | CP-212021 | Qi Caixia, Huawei |
| 920026 | Security Aspects of Enhancements of Support for Edge Computing in 5GC | e E D G E _ 5 G C | S3 | SP-210423 | Bo Zhang, Huawei Technologies |
| 920019 | Edge Computing Management | E C M | S5 | SP-210388 | Deepanshu Gautam, Samsung |
| 880002 | Study on Security Aspects of Enhancement of Support for Edge Computing in 5GC | F S _ e E D G E _ S e c | S3 | SP-200347 | Bo Zhang, Huawei Technologies |
| 930034 | Charging aspects of Edge Computing | E D G E _ C H | S5 | SP-210861 | Yao, Yizhi, Intel |
| 880030 | Study on charging aspects of Edge Computing | F S _ E D G E _ C H | S5 | SP-200467 | Yizhi Yao, Intel |
| 930011 | Charging aspects of Edge Computing | E D G E _ C H | S5 | SP-210861 | Yao, Yizhi, Intel |
| 870029 | Study on enhancements of edge computing management | F S _ e E D G E _ M g t | S5 | SP-200195 | Joey Chou, Intel |
| 0 | Rel-17 Slicing | - |  | - | - |
| 920058 | Rel-17 Enhancements of 3GPP Northbound Interfaces and Application Layer APIs | N B I 1 7 |  | CP-211197 | Yali Yan, Huawei |
| 920013 | CT1 aspects of NBI17 | N B I 1 7 | C1 | CP-211197 | Yali Yan, Huawei |
| 920053 | CT3 aspects of NBI17 | N B I 1 7 | C3 | CP-211197 | Yali Yan, Huawei |
| 920054 | CT4 aspects of NBI17 | N B I 1 7 | C4 | CP-211197 | Yali Yan, Huawei |
| 920012 | Enhancement of 5G PCC related services in Rel-17 | e n 5 G P c c S e r 1 7 | C3 | CP-211193 | Xiaoyun Zhou, Huawei |
| 900032 | Enhancement of Network Slicing Phase 2 | e N S _ P h 2 |  | SP-200976 | ZTE, Jinguo Zhu |
| 850010 | Study on Enhancement of Network Slicing Phase 2 | F S _ e N S _ P h 2 | S2 | SP-190931 | So, Tricci, ZTE |
| 900011 | Stage 2 for Enhancement of Network Slicing Phase 2 | e N S _ P h 2 | S2 | SP-200976 | ZTE, Jinguo Zhu |
| 920059 | Stage 3 for Enhancement of Network Slicing Phase 2 | e N S _ P h 2 | ct | CP-211091 | Hannah Wang, ZTE |
| 910041 | CT1 aspects of eNS_Ph2 | e N S _ P h 2 | C1 | CP-211091 | Hannah Wang, ZTE |
| 910042 | CT3 aspects of eNS_Ph2 | e N S _ P h 2 | C3 | CP-211091 | Hannah Wang, ZTE |
| 910043 | CT4 aspects of eNS_Ph2 | e N S _ P h 2 | C4 | CP-211091 | Hannah Wang, ZTE |
| 911007 | Enhancement of RAN slicing for NR | N R _ s l i c e | R2 | RP-212534 | CMCC |
| 911107 | Core part: Enhancement of RAN slicing for NR | N R _ s l i c e - C o r e | R2 | RP-212534 | CMCC |
| 860022 | Study on network slice management enhancement | F S _ N S M E N | S5 | SP-200766 | Brendan Hassett, Huawei Technologies Sweden AB, |
| 900023 | Study on Charging Aspects for Network Slicing Phase 2 | F S _ N E T S L I C E _ C H _ P h 2 | S5 | SP-201082 | Matrixx, Gerald Görmer |
| 910023 | Study on enhanced security for network slicing Phase 2 | F S _ e N S 2 _ S E C | S3 | SP-210106 | Zander Lei, Huawei |
| 910026 | Study on network slice management capability exposure | F S _ N S C E | S5 | SP-210131 | Xiaobo Yu, Alibaba Group |
| 910095 | Study on 5G NR UE full stack testing for Network Slicing | F S _ N R _ S l i c e _ T e s t | R5 | RP-211977 | CMCC |
| 0 | Rel-17 Multi-(U)SIM | - |  | - | - |
| 840049 | Support for Multi-USIM Devices | M U S I M |  | SP-190309 | Liao, Ellen C, Intel |
| 830019 | Study on MUSIM | F S _ M U S I M | S1 | SP-190091 | Liao, Ellen C, Intel |
| 840040 | Stage 1 of MUSIM | M U S I M | S1 | SP-190309 | Liao, Ellen C, Intel |
| 820012 | Study on Stage 2 (System Enablers) for MUSIM | F S _ M U S I M | S2 | SP-200297 | Sašo Stojanovski, Intel |
| 900013 | System enablers for Multi-USIM devices | M U S I M | S2 | SP-210091 | Sašo Stojanovski, Intel |
| 910015 | CT aspects of MUSIM | M U S I M | ct | CP-212102 | Vivek Gupta, Intel |
| 910063 | CT1 aspects of Enabling Multi-USIM Devices | M U S I M | C1 | CP-212102 | Vivek Gupta, Intel |
| 910064 | CT4 aspects of Enabling Multi-USIM Devices | M U S I M | C4 | CP-212102 | Vivek Gupta, Intel |
| 900017 | Study on the security of the system enablers for devices having multiple Universal Subscriber Identity Modules (USIM) | F S _ M U S I M _ S E C | S3 | SP-201018 | Abhijeet Kolekar, Intel Corporation |
| 860063 | Support for Multi-SIM devices for LTE/NR | L T E _ N R _ M U S I M |  | RP-212610 | Vivo |
| 860163 | Core part: Support for Multi-SIM devices for LTE/NR | L T E _ N R _ M U S I M - C o r e | R2 | RP-212610 | Vivo |
| 0 | Rel-17 other cross-TSG Features | - |  | - | - |
| 930030 | Support of the 5GMSG Service | F S _ 5 G M A R C H | S6 | SP-200835 | Liu, Yue, China Mobile |
| 840036 | Study on support of the 5GMSG Service | F S _ 5 G M A R C H | S6 | SP-200835 | Liu, Yue, China Mobile |
| 890010 | Study on security aspects of the 5GMSG Service (St1 in Rel16) | F S _ S E C _ 5 G M S G | S3 | SP-200878 | Xiaoting Huang, China Mobile |
| 930005 | Security aspects of the 5GMSG Service | 5 G M S G | S3 | SP-210835 | Xiaoting Huang, China Mobile |
| 930040 | Application Architecture for MSGin5G Service | 5 G M A R C H |  | SP-200832 | Liu, Yue, China Mobile |
| 890026 | Application Architecture for MSGin5G Service | 5 G M A R C H | S6 | SP-200832 | Liu, Yue, China Mobile |
| 930004 | CT aspects for enabling MSGin5G Service | 5 G M A R C H | ct | CP-212106 | Liu, Yue, China Mobile |
| 930041 | CT1 aspects for enabling MSGin5G Service | 5 G M A R C H | C1 | CP-212106 | Liu, Yue, China Mobile |
| 930042 | CT3 aspects for enabling MSGin5G Service | 5 G M A R C H | C3 | CP-212106 | Liu, Yue, China Mobile |
| 930002 | IMS voice service support and network usability guarantee for UE s E-UTRA capability disabled scenario in SA 5GS | I N G _ 5 G S | ct | CP-212231 | Mingxue Li, China Telecom |
| 930047 | CT1 aspects of ING_5GS | I N G _ 5 G S | C1 | CP-212231 | Mingxue Li, China Telecom |
| 930046 | CT6 aspects of ING_5GS | I N G _ 5 G S | C6 | CP-212231 | Mingxue Li, China Telecom |
| 930031 | User Consent for 3GPP services | F S _ U C 3 S | S3 | SP-200885 | Rong Wu, Huawei Technologies |
| 890037 | Study on User Consent for 3GPP services | F S _ U C 3 S | S3 | SP-200885 | Rong Wu, Huawei Technologies |
| 930006 | Security aspects on User Consent for 3GPP services | U C 3 S _ S E C | S3 | SP-210836 | Rong Wu, Huawei Technologies |
| 930032 | Improved support for NSA in the service-based management architecture | N S A _ S B M A | S5 | SP-211121 | Lan Zou, Huawei |
| 910031 | Study on Enhancement of service-based management architecture | F S _ e S B M A | S5 | SP-210136 | Lan Zou, Huawei |
| 930009 | Improved support for NSA in the service-based management architecture | N S A _ S B M A | S5 | SP-210858 | Lan Zou, Huawei |
| 930033 | Access control for management service | M S A C | S5 | SP-210859 | Jing Ping, Nokia |
| 890016 | Study on access control for management service | F S _ M N S A C | S5 | SP-200853 | Jing Ping, Nokia |
| 930010 | Access control for management service | M S A C | S5 | SP-210859 | Jing Ping, Nokia |
| 920060 | NPN support of PWS | N P N _ P W S | S1 | SP-210585 | Francesco Pica, Qualcomm |
| 910007 | Start of Pause of Charging via User Plane | S P O C U P | C4 | CP-210076 | YONG, YANG, Ericsson, frank |
| 910030 | File Management | F I M A | S5 | SP-210135 | Pollakowski, Olaf, Nokia |
| 910024 | Enhancements of 3GPP profiles for cryptographic algorithms and security protocols | e C r y p t P r | S3 | SP-210107 | Pinar Comak, Ericsson |
| 920027 | AKMA TLS protocol profiles | A K M A _ T L S | S3 | SP-210424 | Escott, Adrian, Qualcomm |
| 910025 | User Plane Integrity Protection for LTE | U P I P _ S E C _ L T E | S3 | SP-210105 | Evans, Tim, Vodafone |
| 900002 | Restoration of PDN Connections in PGW-C/SMF Set | R P C P S E T | C4 | CP-203020 | LANDAIS, Bruno, Nokia |
| 900037 | Enhancement of Inter-PLMN Roaming | E o I P R | C4 | CP-203021 | Caixia Qi, Huawei |
| 911037 | Reliable Data Service Serialization Indication | R D S S I |  | SP-190446 | Starsinic, Michael, Convida Wireless LLC |
| 840017 | Stage 2 of RDSSI | R D S S I | S2 | SP-190446 | Starsinic, Michael, Convida Wireless LLC |
| 900005 | CT aspects of RDSSI | R D S S I | C1 | CP-203234 | Vivek Gupta, Intel |
| 900021 | Management Aspects of 5G Network Sharing | M A N S | S5 | SP-201080 | Mingrui Sun, China Unicom |
| 920018 | Study on Management Aspects of 5G Network Sharing | F S _ M A N S | S5 | SP-210387 | Mingrui Sun, China Unicom |
| 900019 | Adapting BEST for use in 5G networks | B E S T _ 5 G | S3 | SP-201020 | Keesmaat, Iko, KPN |
| 920049 | Enhanced Service Enabler Architecture Layer for Verticals | e S E A L |  | SP-200987 | Basavaraj (Basu) Pattan, Samsung |
| 900024 | Stage 2 of eSEAL | e S E A L | S6 | SP-200987 | Basavaraj (Basu) Pattan, Samsung |
| 920006 | CT1 aspects of eSEAL | e S E A L | C1 | CP-212098 | Sapan Shah (Samsung Electronics) |
| 920050 | CT3 aspects of eSEAL | e S E A L | C3 | CP-212098 | Sapan Shah (Samsung Electronics) |
| 900033 | Access Traffic Steering, Switch and Splitting support in the 5G system architecture; Phase 2 | A T S S S _ P h 2 |  | SP-200977 | Apostolis Salkintzis, Lenovo |
| 840084 | Study on ATSSS_Ph2 | F S _ A T S S S _ P h 2 | S2 | SP-200095 | So, Tricci, ZTE |
| 900012 | Stage 2 of ATSSS_Ph2 | A T S S S _ P h 2 | S2 | SP-200977 | Apostolis Salkintzis, Lenovo |
| 910013 | CT aspects of ATSSS_Ph2 | A T S S S _ P h 2 | ct | CP-210136 | ZHOU Xingyue (Joy), ZTE |
| 910056 | CT1 aspects of ATSSS_Ph2 | A T S S S _ P h 2 | C1 | CP-210136 | ZHOU Xingyue (Joy), ZTE |
| 910057 | CT3 aspects of ATSSS_Ph2 | A T S S S _ P h 2 | C3 | CP-210136 | ZHOU Xingyue (Joy), ZTE |
| 910058 | CT4 aspects of ATSSS_Ph2 | A T S S S _ P h 2 | C4 | CP-210136 | ZHOU Xingyue (Joy), ZTE |
| 900031 | Enablers for Network Automation for 5G - phase 2 | e N A _ P h 2 |  | SP-200975 | TBD |
| 840022 | Study on Enablers for Network Automation for 5G - phase 2 | F S _ e N A _ P h 2 | S2 | SP-200098 | Xiaobo Wu, Huawei Technologies |
| 890015 | Study on security aspects of enablers for Network Automation (eNA) for 5GS Phase 2 | F S _ e N A _ S E C | S3 | SP-200722 | Xiaoting Huang, China Mobile |
| 900010 | Stage 2 of eNA_Ph2 | e N A _ P h 2 | S2 | SP-200975 | TBD |
| 910012 | CT aspects of eNA_Ph2 | e N A _ P h 2 | ct | CP-211191 | Huang Zhenning (China Mobile) |
| 910088 | CT3 aspects of eNA_Ph2 | e N A _ P h 2 | C3 | CP-211191 | Huang Zhenning (China Mobile) |
| 910089 | CT4 aspects of eNA_Ph2 | e N A _ P h 2 | C4 | CP-211191 | Huang Zhenning (China Mobile) |
| 930007 | Security aspects of eNA_Ph2 | e N A _ P h 2 | S3 | SP-210837 | Chang Liu, China Mobile |
| 900030 | Proximity based Services in 5GS | 5 G _ P r o S e |  | SP-200972 | TBD |
| 830033 | Study on System enhancement for Proximity based Services in 5GS | F S _ 5 G _ P r o S e | S2 | SP-190443 | Qiang Deng, CATT |
| 900007 | Stage 2 for Proximity based Services in 5GS | 5 G _ P r o S e | S2 | SP-201096 | Qiang Deng, CATT |
| 910018 | CT aspects of proximity based services in 5GS | 5 G _ P r o S e | ct | CP-212105 | Yong Jiang, CATT |
| 910072 | CT1 aspects of 5G_ProSe | 5 G _ P r o S e | C1 | CP-212105 | Yong Jiang, CATT |
| 910073 | CT3 aspects of 5G_ProSe | 5 G _ P r o S e | C3 | CP-212105 | Yong Jiang, CATT |
| 910074 | CT4 aspects of 5G_ProSe | 5 G _ P r o S e | C4 | CP-212105 | Yong Jiang, CATT |
| 920081 | CT6 aspects of 5G_ProSe | 5 G _ P r o S e | C6 | CP-212105 | Yong Jiang, CATT |
| 890018 | Study on charging aspects of Proximity-based Services in 5GS | F S _ 5 G _ P r o s e _ C H | S5 | SP-200767 | Shu, Min, CATT |
| 880005 | Study on Security Aspects of Enhancement for Proximity Based Services in 5GS | F S _ 5 G _ P r o S e _ S e c | S3 | SP-200350 | Wei Zhou, CATT |
| 930008 | Security Aspects of Proximity based Services in 5GS | 5 G _ P r o S e | S3 | SP-211120 | Wei Zhou, CATT |
| 911005 | NR Sidelink Relay | N R _ S L _ r e l a y |  | RP-212601 | OPPO |
| 911105 | Core part: NR Sidelink Relay | N R _ S L _ r e l a y - C o r e | R2 | RP-211050 | OPPO |
| 911205 | Perf. part: NR Sidelink Relay | N R _ S L _ r e l a y - P e r f | R4 | RP-211050 | OPPO |
| 890030 | Authentication and key management for applications based on 3GPP credential in 5G | A K M A |  | SP-190711 | Xiaoting Huang, China Mobile |
| 850021 | SA3 aspects of AKMA | A K M A | S3 | SP-190711 | Xiaoting Huang, China Mobile |
| 890008 | CT aspects of AKMA | A K M A - C T | ct | CP-203107 | Huang Zhenning (China Mobile) |
| 890031 | CT1 aspects of AKMA | A K M A - C T | C1 | CP-203107 | Huang Zhenning (China Mobile) |
| 890032 | CT3 aspects of AKMA | A K M A - C T | C3 | CP-203107 | Huang Zhenning (China Mobile) |
| 890033 | CT4 aspects of AKMA | A K M A - C T | C4 | CP-203107 | Huang Zhenning (China Mobile) |
| 890049 | Enhancement of Private Network support for NG-RAN | N G _ R A N _ P R N _ e n h | R3 | RP-212585 | China Telecom |
| 890149 | Core part: Enhancement of Private Network support for NG-RAN | N G _ R A N _ P R N _ e n h - C o r e | R3 | RP-212585 | China Telecom |
| 840046 | Multimedia Priority Service (MPS) Phase 2 | M P S 2 |  | SP-190305 | Singh, Ray P; Perspecta Labs |
| 780005 | Study on MPS2 | F S _ M P S 2 | S1 | SP-190315 | Singh, Ray P; Vencore Labs |
| 850011 | Study on MPS2, Stage 2 | F S _ M P S 2 _ S t 2 | S2 | SP-190629 | Don Lukacs; Perspecta Labs |
| 840032 | Stage 1 of MPS2 | M P S 2 | S1 | SP-190561 | Singh, Ray P; Perspecta Labs |
| 870002 | Stage 2 of MPS2 | M P S 2 | S2 | SP-200519 | Streijl, Robert, Perspecta Labs |
| 880031 | Stage 3 of MPS2 | M P S 2 | ct | CP-201207 | Peter Monnes, Perspecta Labs |
| 880050 | CT1 aspects of MPS2 | M P S 2 | C1 | CP-201207 | Peter Monnes, Perspecta Labs |
| 880058 | CT3 aspects of MPS2 | M P S 2 | C3 | CP-201207 | Peter Monnes, Perspecta Labs |
| 880051 | CT4 aspects of MPS2 | M P S 2 | C4 | CP-201207 | Peter Monnes, Perspecta Labs |
| 890011 | Mission critical security enhancements phase 2 | M C X S e c 2 | S3 | SP-200879 | Woodward, Tim, Motorola Solutions, Inc, |
| 890070 | PAP/CHAP protocols usage in 5GS | P A P _ C H A P | ct | CP-202251 | Mingxue Li, China Telecom |
| 890071 | CT1 aspects of PAP/CHAP protocols usage in 5GS | P A P _ C H A P | C1 | CP-202251 | Mingxue Li, China Telecom |
| 890072 | CT3 aspects of PAP/CHAP protocols usage in 5GS | P A P _ C H A P | C3 | CP-210188 | Mingxue Li, China Telecom |
| 890001 | Service-based support for SMS in 5GC | S M S _ S B I |  | CP-212023 | HAOUARI, Wafa, Orange |
| 890028 | CT1 aspects of SMS_SBI | S M S _ S B I | C1 | CP-212023 | HAOUARI, Wafa, Orange |
| 890029 | CT4 aspects of SMS_SBI | S M S _ S B I | C4 | CP-212023 | HAOUARI, Wafa, Orange |
| 840048 | Enhanced Relays for Energy eFficiency and Extensive Coverage | R E F E C |  | SP-190307 | Norp, Toon, KPN |
| 810018 | Study on REFEC | F S _ R E F E C | S1 | SP-180785 | Jose Luis Almodovar Chico, KPN |
| 840034 | Stage 1 of REFEC | R E F E C | S1 | SP-190307 | Norp, Toon, KPN |
| 840044 | Network Controlled Interactive Service | N C I S |  | SP-190303 | Ning YANG, oppo |
| 800015 | Study on NCIS | F S _ N C I S | S1 | SP-180341 | YANG Ning, Oppo |
| 840030 | Stage 1 of NCIS | N C I S | S1 | SP-191039 | Ning YANG, oppo |
| 840019 | 5G System Enhancement for Advanced Interactive Services | 5 G _ A I S | S2 | SP-190564 | Lei Yixue, Tencent |
| 890009 | Points for 8K VR 360 Video over 5G | 8 K _ V R _ 5 G | S4 | SP-200667 | Ozgur Oyman, Company: Intel |
| 880043 | Multi-Device and multi-identity Enhancements (MuDE) | M u D E |  | SP-190823 | Adrian.buckley@vivo.com |
| 850041 | Stage 1 of MuDE | M u D E | S1 | SP-190823 | Adrian.buckley@vivo.com |
| 880018 | Stage 3 of MuDE | M u D e | C1 | CP-201162 | Adrian Buckley |
| 920055 | Best Practice of Packet Forwarding Control Protocol (PFCP) | B E P o P | ct | CP-212024 | Song Yue, China Mobile |
| 920056 | CT3 aspects of BEPoP | B E P o P | C3 | CP-212024 | Song Yue, China Mobile |
| 880014 | CT4 aspects of BEPoP | B E P o P | C4 | CP-212024 | Song Yue, China Mobile |
| 880029 | Enhancement of Handover Optimization | E _ H O O | S5 | SP-200466 | Elmdahl, Per, Ericsson |
| 880028 | Management data collection control and discovery | M A D C O L | S5 | SP-200465 | Pollakowski, Olaf, Nokia |
| 880026 | Management of the enhanced tenant concept | e M E M T A N E | S5 | SP-200463 | Zhu, Lei, Huawei |
| 880025 | Enhancements of 5G performance measurements and KPIs | e P M _ K P I _ 5 G | S5 | SP-200462 | Yizhi Yao, Intel |
| 880012 | Extension for headset interface tests of UE | H I n T | S4 | SP-200398 | Jan Reimes, HEAD acoustics GmbH, |
| 840038 | MC services support on IOPS mode of operation | M C I O P S | S6 | SP-190944 | Camilo.solano@ericsson.com |
| 840016 | Supporting Flexible Local Area Data Network | F L A D N | S2 | SP-190563 | LG Electronics, Hyunsook Kim |
| 820003 | Support of Immersive Teleconferencing and Telepresence for Remote Terminals | I T T 4 R T | S4 | SP-180985 | Ozgur Oyman, Intel |
| 830005 | Terminal Audio quality performance and Test methods for Immersive Audio Services | A T I A S | S4 | SP-190040 | Stéphane Ragot, Orange |
| 860012 | Handsets Featuring Non-Traditional Earpieces | H a N T E | S4 | SP-190989 | Schevciw, Andre, Qualcomm |
| 920008 | 5GMS AF Event Exposure | E V E X | S4 | SP-210374 | Lo, Charles, Qualcomm Inc., |
| 920009 | Edge Extensions to the 5G Media Streaming Architecture | 5 G M S _ E D G E | S4 | SP-210375 | Bouazizi, Imed, Qualcomm Inc. |
| 920010 | 5G Multicast-Broadcast User Service Architecture and related 5GMS Extensions | 5 M B U S A | S4 | SP-210376 | TAN, PENG, TELUS |
| 920011 | 8K Television over 5G | 8 K _ T V _ 5 G | S4 | SP-210381 | Thomas Stockhammer |
| 830017 | Verification-modified Calling Name Display | V M O D _ D I S P L A Y | S1 | SP-190085 | Hala Mowafy, Ericsson |
| 850038 | Enhanced Calling Name Service Analytics Interworking | e C N A M _ A n | S1 | SP-190940 | Hala.Mowafy@ericsson.com |
| 880049 | Enhancement for the 5G Control Plane Steering of Roaming for UE in Connected mode | e C P S O R _ C O N |  | CP-210148 | Minokuchi, Atsushi, NTT DOCOMO |
| 850039 | Stage 1 of eCPSOR_CON | e C P S O R _ C O N | S1 | SP-190941 | Minokuchi, Atsushi, NTT DOCOMO |
| 880022 | Stage 3 (CT1) of eCPSOR_CON | e C P S O R _ C O N | C1 | CP-210148 | Al-Bakri Ban, NTT DOCOMO |
| 900036 | Stage 3 (CT4) of eCPSOR_CON | e C P S O R _ C O N | C4 | CP-210148 | Al-Bakri Ban, NTT DOCOMO |
| 920061 | Stage 3 (CT6) of eCPSOR_CON | e C P S O R _ C O N | C6 | CP-210148 | Al-Bakri Ban, NTT DOCOMO |
| 910090 | Integration of Generic Bootstrapping Architecture (GBA) into 5GC | G B A _ 5 G |  | SP-190714 | Vlasios Tsiatsis, Ericsson |
| 850023 | Security aspects of Integration of GBA into 5GC | G B A _ 5 G | S3 | SP-190714 | Vlasios Tsiatsis, Ericsson |
| 910004 | CT aspects of Integration of GBA into SBA | G B A _ 5 G | C4 | CP-210283 | de Gregorio, Jesús (Ericsson) |
| 910047 | Integration of Generic Bootstrapping Architecture (GBA) into 5GC | G B A _ 5 G | S3 | SP-190714 | Vlasios Tsiatsis, Ericsson |
| 770024 | EVS Codec Extension for Immersive Voice and Audio Services | I V A S _ C o d e c | S4 | SP-170611 | Bin Wang, Huawei Technologies Co Ltd |
| 850045 | Support for Minimization of service Interruption | M I N T |  | SP-190814 | SungDuck Chun; LG Electronics |
| 830018 | Study on MINT | F S _ M I N T | S1 | SP-190090 | SungDuck Chun; LG Electronics |
| 850036 | Stage 1 of MINT | M I N T | S1 | SP-190938 | SungDuck Chun; LG Electronics |
| 920062 | Stage 2 of MINT | M I N T | S2 | SP-210582 | Hyunsook Kim, LG Electronics |
| 900004 | Study on the CT aspects of Support for Minimization of service Interruption | F S _ M I N T - C T | C1 | CP-203232 | Sang Min Park, LG Electronics |
| 930003 | Stage 3 of MINT | M I N T | ct | CP-212166 | Sang Min Park |
| 930049 | CT1 aspects of MINT | M I N T | C1 | CP-212166 | Sang Min Park |
| 930045 | CT4 aspects of MINT | M I N T | C4 | CP-212166 | Sang Min Park |
| 930048 | CT6 aspects of MINT | M I N T | C6 | CP-212166 | Sang Min Park |
| 860008 | IMS emergency support for Stand-alone Non-Public Network (SNPN) | I E S N P N | S1 | SP-191038 | Wong, Curt; |
| 870023 | Management of non-public networks (NPN) | O A M _ N P N | S5 | SP-200189 | ZHANG, Kai, Huawei |
| 860016 | Assurance Specification for IMS | S C A S _ I M S | S3 | SP-191128 | Bo Zhang, Huawei Technologies |
| 870020 | Security Assurance Specification for 5G (eSCAS_5G) | e S C A S _ 5 G | S3 | SP-200149 | Rong Wu, Huawei Technologies Co. |
| 890013 | eSCAS_5G for Network Slice-Specific Authentication and Authorization Function (NSSAAF) | S C A S _ 5 G _ N S S A A F | S3 | SP-200720 | Rong Wu, Huawei Technologies Co., Ltd. |
| 870017 | eSCAS_5G for Non-3GPP InterWorking Function | S C A S _ 5 G _ N 3 I W F | S3 | SP-200146 | Feng Gao, China Unicom |
| 870018 | eSCAS_5G for 5G NWDAF | S C A S _ 5 G _ N W D A F | S3 | SP-200147 | QI Minpeng, China Mobile |
| 870019 | eSCAS_5G for Service Communication Proxy | S C A S _ 5 G _ S E C O P | S3 | SP-200148 | Wei Lu, Nokia |
| 880003 | eSCAS_5Gfor Inter PLMN UP Security | S C A S _ 5 G _ I P U P S | S3 | SP-200348 | Jin PENG, ZTE Corporation |
| 870022 | Enhancements on EE for 5G networks | E E 5 G P L U S | S5 | SP-200188 | Jean-Michel CORNILY, ORANGE |
| 870024 | Enhancement on Management Aspects of 5G Service-Level Agreement | E M A 5 S L A | S5 | SP-210860 | SHI, Xiaonan, CMCC |
| 870025 | Management of MDT enhancement in 5G | e _ 5 G M D T | S5 | SP-200191 | Ayani, Zhulia, Ericsson |
| 870026 | Additional NRM features | a d N R M | S5 | SP-200192 | Jing Ping, Nokia |
| 870027 | Enhancement of QoE Measurement Collection | e Q o E | S5 | SP-200193 | Petersen, Robert, Ericsson |
| 860061 | Study on NR QoE (Quality of Experience) management and optimizations for diverse services | F S _ N R _ Q o E | R3 | RP-193256 | China Unicom |
| 870030 | Enhanced Closed loop SLS Assurance | e C O S L A | S5 | SP-200196 | Groenendijk, Jan, Ericsson |
| 870034 | Enhanced Mission Critical Push-to-talk architecture phase 3 | e n h 3 M C P T T |  | SP-200108 | Dom Lazara; Motorola Solutions |
| 870016 | Stage 2 aspects of enh3MCPTT | e n h 3 M C P T T | S6 | SP-200108 | Dom Lazara; Motorola Solutions |
| 900039 | Stage 3 aspects of enh3MCPTT | e n h 3 M C P T T - C T | C1 | CP-202188 |  |
| 840028 | IMS Charging in 5G System Architecture | 5 G S I M S C H | S5 | SP-190367 | Jahangir, Zeeshan, T-Mobile USA |
| 810027 | Intent driven management service for mobile network | I D M S _ M N | S5 | SP-180899 | Zou Lan, Huawei |
| 820006 | Study on User Plane Integrity Protection | F S _ U P _ I P _ S e c | S3 | SP-181035 | Evans, Tim, Vodafone |
| 890012 | Enhancements to User Plane Integrity Protection Support in 5GS | e U P I P _ S E C | S3 | SP-200719 | Anand Palanigounder, Qualcomm |
| 820035 | Discovery of management services in 5G | 5 G D M S | S5 | SP-200770 | Brendan Hassett, Huawei Technologies Sweden AB |
| 860024 | Network policy managementfor 5G mobile networks | N P M | S5 | SP-191211 | China Mobile Shasha Guo |
| 860025 | Lawful Interception Rel-17 | L I 1 7 | S3 | SP-190983 | Alex Leadbeater, BT |
| 850047 | (Small) Technical Enhancements and Improvements for Rel-17 | T E I 1 7 |  |  |  |
| 870003 | IMS Optimization for HSS Group ID in an SBA environment | T E I 1 7 _ I M S G I D | S2 | SP-200452 | Foti, George, Ericsson |
| 910044 | Same PCF Selection For AMF and SMF | T E I 1 7 _ S P S F A S |  | SP-200447 | Heng, Nie, China Telecom |
| 880016 | Stage 2 for TEI17_SPSFAS | T E I 1 7 _ S P S F A S | S2 | SP-200447 | Heng, Nie, China Telecom |
| 910003 | CT aspects on TEI17_SPSFAS | T E I 1 7 _ S P S F A S | ct | CP-211184 | Yue Sun, China Telecom |
| 910045 | CT3 aspects on TEI17_SPSFAS | T E I 1 7 _ S P S F A S | C3 | CP-211184 | Yue Sun, China Telecom |
| 910046 | CT4 aspects on TEI17_SPSFAS | T E I 1 7 _ S P S F A S | C4 | CP-211184 | Yue Sun, China Telecom |
| 930036 | System enhancement for Redundant PDU Session | T E I 1 7 _ S E _ R P S |  | SP-200448 | Devaki Chandramouli |
| 880017 | Stage 2 for TEI17_SE_RPS | T E I 1 7 _ S E _ R P S | S2 | SP-200448 | Devaki Chandramouli |
| 930001 | CT aspects of TEI17_SE_RPS | T E I 1 7 _ S E _ R P S | ct | CP-212099 | WON, Sung Hwan, Nokia |
| 930037 | CT1 aspects of TEI17_SE_RPS | T E I 1 7 _ S E _ R P S | C1 | CP-212099 | WON, Sung Hwan, Nokia |
| 930038 | CT3 aspects of TEI17_SE_RPS | T E I 1 7 _ S E _ R P S | C3 | CP-212099 | WON, Sung Hwan, Nokia |
| 930039 | CT4 aspects of TEI17_SE_RPS | T E I 1 7 _ S E _ R P S | C4 | CP-212099 | WON, Sung Hwan, Nokia |
| 910078 | Support for Signed Attestation for Priority and Emergency Sessions | T E I 1 7 _ S A P E S |  | SP-200453 | Foti, George, Ericsson |
| 870004 | Stage 2 for TEI17_SAPES | T E I 1 7 _ S A P E S | S2 | SP-200453 | Foti, George, Ericsson |
| 910020 | CT aspects on TEI17_SAPES | T E I 1 7 _ S A P E S | ct | CP-210272 | Axell, Jörgen, Ericsson |
| 910079 | CT1 aspects on TEI17_SAPES | T E I 1 7 _ S A P E S | C1 | CP-210272 | Axell, Jörgen, Ericsson |
| 910080 | CT3 aspects on TEI17_SAPES | T E I 1 7 _ S A P E S | C3 | CP-210272 | Axell, Jörgen, Ericsson |
| 910081 | Dynamically Changing AM Policies in the 5GC | T E I 1 7 _ D C A M P |  | SP-200446 | Pancorbo, Belen (Ericsson) |
| 870005 | Stage 2 for TEI17_DCAMP | T E I 1 7 _ D C A M P | S2 | SP-200446 | Pancorbo, Belen (Ericsson) |
| 910009 | CT aspects on TEI17_DCAMP | T E I 1 7 _ D C A M P | ct | CP-212163 | Garcia Azorero, Fuencisla. Ericsson |
| 910082 | CT3 aspects on TEI17_DCAMP | T E I 1 7 _ D C A M P | C3 | CP-212163 | Garcia Azorero, Fuencisla. Ericsson |
| 910083 | CT4 aspects on TEI17_DCAMP | T E I 1 7 _ D C A M P | C4 | CP-212163 | Garcia Azorero, Fuencisla. Ericsson |
| 870006 | IP address pool information from UDM | T E I 1 7 _ I P U | S2 | SP-200454 | Stefan Rommer |
| 910084 | Dynamic management of group-based event monitoring | T E I 1 7 _ G E M |  | SP-200455 | Qian XB Chen, Ericsson |
| 870007 | Stage 2 for TEI17_GEM | T E I 1 7 _ G E M | S2 | SP-200455 | Qian XB Chen, Ericsson |
| 910010 | CT aspects on TEI17_GEM | T E I 1 7 _ G E M | ct | CP-212165 | Maria Tianmei, Liang, Ericsson |
| 910085 | CT3 aspects on TEI17_GEM | T E I 1 7 _ G E M | C3 | CP-212165 | Maria Tianmei, Liang, Ericsson |
| 910086 | CT4 aspects on TEI17_GEM | T E I 1 7 _ G E M | C4 | CP-212165 | Maria Tianmei, Liang, Ericsson |
| 920041 | Support of different slices over different Non 3GPP access | T E I 1 7 _ N 3 S L I C E |  | SP-200456 | Laurent Thiebaut, Nokia |
| 870008 | Stage 2 for TEI17_N3SLICE | T E I 1 7 _ N 3 S L I C E | S2 | SP-200456 | Laurent Thiebaut, Nokia |
| 930099 | CT1 aspects of TEI17_N3SLICE | T E I 1 7 _ N 3 S L I C E | C1 | CP-211089 | LANDAIS, Bruno, Nokia |
| 920001 | CT4 aspects of TEI17_N3SLICE | T E I 1 7 _ N 3 S L I C E | C4 | CP-211089 | LANDAIS, Bruno, Nokia |
| 910087 | N7/N40 Interfaces Enhancements to Support GERAN and UTRAN | T E I 1 7 _ N I E S G U |  | SP-200457 | Aihua, Li, China Mobile |
| 870009 | Stage 2 for TEI17_NIESGU | T E I 1 7 _ N I E S G U | S2 | SP-200457 | Aihua, Li, China Mobile |
| 890019 | SA5 aspects of N40 Interface Enhancements to Support GERAN and UTRAN | T E I 1 7 _ N I E S G U | S5 | SP-200854 | Dong, Jia, China Mobile |
| 910011 | CT3 aspects of N7 Interfaces Enhancements to Support GERAN and UTRAN | T E I 1 7 _ N I E S G U | C3 | CP-211194 | Huang Zhenning (China Mobile) |
| 920082 | CT4 aspects of N7 Interfaces Enhancements to Support GERAN and UTRAN | T E I 1 7 _ N I E S G U | C4 | CP-211194 | Huang Zhenning (China Mobile) |
| 880019 | Stage-3 5GS NAS protocol development 17 | 5 G P r o t o c 1 7 | C1 | CP-201163 | Sedlacek, Ivo, Ericsson |
| 880044 | Stage-3 5GS NAS protocol development 17 general aspects | 5 G P r o t o c 1 7 | C1 | CP-201163 | Sedlacek, Ivo, Ericsson |
| 880045 | Stage-3 5GS NAS protocol development 17 non-IETF aspects | 5 G P r o t o c 1 7 - n o n 3 G P P | C1 | CP-201163 | Sedlacek, Ivo, Ericsson |
| 880021 | Stage-3 SAE Protocol Development | S A E S 1 7 | C1 | CP-201165 | Aghili, Behrouz, InterDigital Communications |
| 880046 | Stage-3 SAE Protocol Development general aspects | S A E S 1 7 | C1 | CP-201165 | Aghili, Behrouz, InterDigital Communications |
| 880047 | Stage-3 SAE Protocol Development CSFB aspects | S A E S 1 7 - C S F B | C1 | CP-201165 | Aghili, Behrouz, InterDigital Communications |
| 880048 | Stage-3 SAE Protocol Development non 3GPP aspects | S A E S 1 7 - n o n 3 G P P | C1 | CP-201165 | Aghili, Behrouz, InterDigital Communications |
| 911034 | IMS Stage-3 IETF Protocol Alignment | I M S P r o t o c 1 7 | ct | CP-201167 | Gkatzikis, Lazaros(Nokia) |
| 880023 | CT1 aspects of IMS Stage-3 IETF Protocol Alignment | I M S P r o t o c 1 7 | C1 | CP-201167 | Gkatzikis, Lazaros(Nokia) |
| 911035 | CT3 aspects of IMS Stage-3 IETF Protocol Alignment | I M S P r o t o c 1 7 | C3 | CP-201167 | Gkatzikis, Lazaros(Nokia) |
| 880032 | Packet Flow Description (PFD) management enhancement | p f d M a n E n h | C3 | CP-210183 | Xiaoyun Zhou, Huawei |
| 880013 | Service Based Interface Protocol Improvements Release 17 | S B I P r o t o c 1 7 |  | CP-211088 | Song Yue, China Mobile |
| 880053 | CT3 aspects of SBIProtoc17 | S B I P r o t o c 1 7 | C3 | CP-211088 | Song Yue, China Mobile |
| 880054 | CT4 aspects of SBIProtoc17 | S B I P r o t o c 1 7 | C4 | CP-211088 | Song Yue, China Mobile |
| 850028 | Study on enhancement of Management Data Analytics Service | F S _ e M D A S | S5 | SP-190930 | Yao, Yizhi, Intel |
| 910027 | Enhancements of Management Data Analytics Service | e M D A S | S5 | SP-210132 | Yao, Yizhi, Intel |
| 0 | Rel-17 Other NR-only improvements | - |  | - | - |
| 911008 | NR QoE magement and optimizations for diverse services | N R _ Q o E |  | RP-211406 | China Unicom |
| 911108 | Core part: NR QoE management and optimizations for diverse services | N R _ Q o E - C o r e | R3 | RP-211406 | China Unicom |
| 911009 | Rel-17 Power Class 2 UE for NR inter-band CA with or without SUL configurations with x (16>=x>2) bands DL and y (y=1, 2) bands UL | N R _ U E _ P C 2 _ R 1 7 _ C A D C _ S U L _ x B D L _ y B U L |  | RP-212182 | Huawei |
| 911109 | Core part: NR_UE_PC2_R17_CADC_SUL_xBDL_yBUL | N R _ U E _ P C 2 _ R 1 7 _ C A D C _ S U L _ x B D L _ y B U L - C o r e | R4 | RP-212182 | Huawei |
| 911209 | Perf. part: NR_UE_PC2_R17_CADC_SUL_xBDL_yBUL | N R _ U E _ P C 2 _ R 1 7 _ C A D C _ S U L _ x B D L _ y B U L - P e r f | R4 | RP-212182 | Huawei |
| 911010 | Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC) | N R _ F R 1 _ T R P _ T R S |  | RP-212030 | Vivo |
| 911110 | Core part: NR_FR1_TRP_TRS | N R _ F R 1 _ T R P _ T R S - C o r e | R4 | RP-212030 | Vivo |
| 911210 | Perf. part: NR_FR1_TRP_TRS | N R _ F R 1 _ T R P _ T R S - P e r f | R4 | RP-212030 | Vivo |
| 911012 | High power UE (power class 1.5) for NR band n79 | N R _ U E _ P C 1 _ 5 _ n 7 9 |  | RP-210843 | CMCC |
| 911112 | Core part: NR_UE_PC1_5_n79 | N R _ U E _ P C 1 _ 5 _ n 7 9 - C o r e | R4 | RP-210843 | CMCC |
| 911212 | Perf. part: NR_UE_PC1_5_n79 | N R _ U E _ P C 1 _ 5 _ n 7 9 - P e r f | R4 | RP-210843 | CMCC |
| 930052 | UE Conformance - High power UE (power class 1.5) for NR band n79 | N R _ U E _ P C 1 _ 5 _ n 7 9 - U E C o n T e s t | R5 | RP-211979 | CMCC |
| 911013 | High power UE (power class 2) for NR band n34 | N R _ U E _ P C 2 _ n 3 4 |  | RP-210844 | CMCC |
| 911113 | Core part: NR_UE_PC2_n34 | N R _ U E _ P C 2 _ n 3 4 - C o r e | R4 | RP-210844 | CMCC |
| 911213 | Perf. part: NR_UE_PC2_n34 | N R _ U E _ P C 2 _ n 3 4 - P e r f | R4 | RP-210844 | CMCC |
| 930053 | UE Conformance - High power UE (power class 2) for NR band n34 | N R _ U E _ P C 2 _ n 3 4 - U E C o n T e s t | R5 | RP-211980 | CMCC |
| 911014 | High power UE (power class 2) for NR band n39 | N R _ U E _ P C 2 _ n 3 9 |  | RP-210845 | CMCC |
| 911114 | Core part: NR_UE_PC2_n39 | N R _ U E _ P C 2 _ n 3 9 - C o r e | R4 | RP-210845 | CMCC |
| 911214 | Perf. part: NR_UE_PC2_n39 | N R _ U E _ P C 2 _ n 3 9 - P e r f | R4 | RP-210845 | CMCC |
| 930054 | UE Conformance - High power UE (power class 2) for NR band n39 | N R _ U E _ P C 2 _ n 3 9 - U E C o n T e s t | R5 | RP-211981 | CMCC |
| 911015 | Introduction of FR2 FWA (Fixed Wireless Access) UE with maximum TRP of 23dBm for band n259 | N R _ F R 2 _ F W A _ B n 2 5 9 |  | RP-210875 | SoftBank Corp., KDDI, NTT DOCOMO, Rakuten Mobile, Qualcomm |
| 911115 | Core part: NR_FR2_FWA_Bn259 | N R _ F R 2 _ F W A _ B n 2 5 9 - C o r e | R4 | RP-210875 | SoftBank Corp., KDDI, NTT DOCOMO, Rakuten Mobile, Qualcomm |
| 911215 | Perf. part: NR_FR2_FWA_Bn259 | N R _ F R 2 _ F W A _ B n 2 5 9 - P e r f | R4 | RP-210875 | SoftBank Corp., KDDI, NTT DOCOMO, Rakuten Mobile, Qualcomm |
| 911006 | NR Uplink Data Compression (UDC) | N R _ U D C |  | RP-211203 | CATT |
| 911106 | Core part: NR_UDC | N R _ U D C - C o r e | R2 | RP-211203 | CATT |
| 910097 | Study on optimizations of pi/2 BPSK uplink power in NR | F S _ N R _ O p t _ p i 2 B P S K | R4 | RP-210910 | Huawei |
| 910096 | Study on band combination handling in RAN4 | F S _ N R _ E N D C _ c o m b o _ r u l e s | R4 | RP-211146 | ZTE |
| 880078 | Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) requirements for NR UEs | N R _ M I M O _ O T A |  | RP-201800 | CAICT |
| 880178 | Core part: NR_MIMO_OTA | N R _ M I M O _ O T A - C o r e | R4 | RP-201800 | CAICT |
| 880278 | Perf. Part: NR_MIMO_OTA | N R _ M I M O _ O T A - P e r f | R4 | RP-201800 | CAICT |
| 870064 | Introduction of FR2 FWA (Fixed Wireless Access) UE with maximum TRP (Total Radiated Power) of 23dBm for band n257 and n258 | N R _ F R 2 _ F W A _ B n 2 5 7 _ B n 2 5 8 |  | RP-202565 | Softbank |
| 870164 | Core part: NR_FR2_FWA_Bn257_Bn258 | N R _ F R 2 _ F W A _ B n 2 5 7 _ B n 2 5 8 - C o r e | R4 | RP-202565 | Softbank |
| 870264 | Perf. Part: NR_FR2_FWA_Bn257_Bn258 | N R _ F R 2 _ F W A _ B n 2 5 7 _ B n 2 5 8 - P e r f | R4 | RP-202565 | Softbank |
| 880092 | Adding channel bandwidth support to existing NR bands | N R _ b a n d s _ R 1 7 _ B W s |  | RP-212531 | Ericsson |
| 880192 | Core part: NR_bands_R17_BWs | N R _ b a n d s _ R 1 7 _ B W s - C o r e | R4 | RP-212531 | Ericsson |
| 880093 | Introduction of channel bandwidths 35MHz and 45MHz for NR FR1 | N R _ F R 1 _ 3 5 M H z _ 4 5 M H z _ B W |  | RP-211386 | Huawei |
| 880193 | Core part: NR_FR1_35MHz_45MHz_BW | N R _ F R 1 _ 3 5 M H z _ 4 5 M H z _ B W - C o r e | R4 | RP-211386 | Huawei |
| 880293 | Perf. Part: NR_FR1_35MHz_45MHz_BW | N R _ F R 1 _ 3 5 M H z _ 4 5 M H z _ B W - P e r f | R4 | RP-211386 | Huawei |
| 880097 | SAR schemes for UE power class 2 (PC2) for NR inter-band Carrier Aggregation and supplemental uplink (SUL) configurations with 2 bands UL | N R _ S A R _ P C 2 _ i n t e r B _ S U L _ 2 B U L |  | RP-212530 | China Telecom |
| 880197 | Core part: NR_SAR_PC2_interB_SUL_2BUL | N R _ S A R _ P C 2 _ i n t e r B _ S U L _ 2 B U L - C o r e | R4 | RP-212530 | China Telecom |
| 880297 | Perf. Part: NR_SAR_PC2_interB_SUL_2BUL | N R _ S A R _ P C 2 _ i n t e r B _ S U L _ 2 B U L - P e r f | R4 | RP-212530 | China Telecom |
| 920065 | UE Conformance - SAR schemes for UE power class 2 (PC2) for NR inter-band Carrier Aggregation and supplemental uplink (SUL) configurations with 2 bands UL | N R _ S A R _ P C 2 _ i n t e r B _ S U L _ 2 B U L - U E C o n T e s t | R5 | RP-211139 | China Telecom |
| 900061 | NR coverage enhancements | N R _ c o v _ e n h |  | RP-211566 | China Telecom |
| 860036 | Study on on NR coverage enhancements | F S _ N R _ c o v _ e n h | R1 | RP-200861 | China Telecom |
| 900161 | Core part: NR_cov_enh | N R _ c o v _ e n h - C o r e | R1 | RP-211566 | China Telecom |
| 900261 | Perf. part: NR_cov_enh | N R _ c o v _ e n h - P e r f | R4 | RP-211566 | China Telecom |
| 900062 | Support of reduced capability NR devices | N R _ r e d c a p |  | RP-211574 | Ericsson |
| 860035 | Study on support of reduced capability NR devices | F S _ N R _ r e d c a p | R1 | RP-202704 | Ericsson |
| 900162 | Core part: NR_redcap | N R _ r e d c a p - C o r e | R1 | RP-211574 | Ericsson |
| 900262 | Perf. part: NR_redcap | N R _ r e d c a p - P e r f | R4 | RP-211574 | Ericsson |
| 930018 | Architecture Enhancement for NR Reduced Capability Devices | A R C H _ N R _ R E D C A P | S2 | SP-211100 | Aihua Li, China Mobile |
| 900064 | Additional NR bands for UL-MIMO power class 3 (PC3) in Rel-17 | N R _ b a n d s _ U L _ M I M O _ P C 3 _ R 1 7 |  | RP-212184 | Huawei |
| 900164 | Core part: NR_bands_UL_MIMO_PC3_R17 | N R _ b a n d s _ U L _ M I M O _ P C 3 _ R 1 7 - C o r e | R4 | RP-212184 | Huawei |
| 900264 | Perf. part: NR_bands_UL_MIMO_PC3_R17 | N R _ b a n d s _ U L _ M I M O _ P C 3 _ R 1 7 - P e r f | R4 | RP-212184 | Huawei |
| 900167 | Introduction of bandwidth combination set 4 (BCS4) for NR | N R _ B C S 4 - C o r e | R4 | RP-202832 | Ericsson |
| 900068 | Downlink interruption for NR and EN-DC band combinations to conduct dynamic Tx Switching in Uplink | D L _ i n t r p t _ c o m b o s _ T x S W _ R 1 7 |  | RP-210478 | China Telecom |
| 900168 | Core part: DL_intrpt_combos_TxSW_R17 | D L _ i n t r p t _ c o m b o s _ T x S W _ R 1 7 - C o r e | R4 | RP-202885RP-210478 | China Telecom |
| 900268 | Perf. part: DL_intrpt_combos_TxSW_R17 | D L _ i n t r p t _ c o m b o s _ T x S W _ R 1 7 - P e r f | R4 | RP-202885RP-210478 | China Telecom |
| 900069 | High-power UE (power class 1.5) operation in NR bands n77 and n78 | H P U E _ P C 1 _ 5 _ n 7 7 _ n 7 8 |  | RP-202912 | Qualcomm |
| 900169 | Core part: HPUE_PC1_5_n77_n78 | H P U E _ P C 1 _ 5 _ n 7 7 _ n 7 8 - C o r e | R4 | RP-202912 | Qualcomm |
| 930055 | UE Conformance - High-power UE (power class 1.5) operation in NR bands n77 and n78 | H P U E _ P C 1 _ 5 _ n 7 7 _ n 7 8 - U E C o n T e s t | R5 | RP-212581 | Verizon |
| 900070 | NR repeaters | N R _ r e p e a t e r s |  | RP-212129 | Qualcomm |
| 900170 | Core part: HPUE_PC1_5_n77_n78 | N R _ r e p e a t e r s - C o r e | R4 | RP-212129 | Qualcomm |
| 930250 | Perf. part: NR repeaters | N R _ r e p e a t e r s - P e r f | R4 | RP-212129 | Qualcomm |
| 930056 | Increasing UE power high limit for CA and DC | P o w e r _ L i m i t _ C A _ D C | R4 | RP-212622 | Qualcomm |
| 930156 | Core part: Increasing UE power high limit for CA and DC | P o w e r _ L i m i t _ C A _ D C - C o r e | R4 | RP-212622 | Qualcomm |
| 930057 | Introduction of upper 700MHz A block E-UTRA band for the US | L T E _ u p p e r _ 7 0 0 M H z _ A | R4 | RP-212618 | Puloli |
| 930157 | Core part: Introduction of upper 700MHz A block E-UTRA band for the US | L T E _ u p p e r _ 7 0 0 M H z _ A - C o r e | R4 | RP-212618 | Puloli |
| 930257 | Perf. part: Introduction of upper 700MHz A block E-UTRA band for the US | L T E _ u p p e r _ 7 0 0 M H z _ A - P e r f | R4 | RP-212618 | Puloli |
| 930058 | High power UE (power class 2) for one NR FDD band | N R _ P C 2 _ U E _ F D D | R4 | RP-212633 | China Unicom |
| 930158 | Core part: High power UE (power class 2) for one NR FDD band | N R _ P C 2 _ U E _ F D D - C o r e | R4 | RP-212633 | China Unicom |
| 930258 | Perf. part: High power UE (power class 2) for one NR FDD band | N R _ P C 2 _ U E _ F D D - P e r f | R4 | RP-212633 | China Unicom |
| 0 | Rel-17 NR Carrier Aggregation | - |  | - | - |
| 0 | Rel-17 NR intra band Carrier Aggregation (itrabCA) | - |  | - | - |
| 881005 | NR itrabCA for xCC DL/yCC UL including contiguous and non-contiguous spectrum (x>=y) | N R _ C A _ R 1 7 _ I n t r a |  | RP-211757 | Ericsson |
| 881105 | Core part: NR_CA_R17_Intra | N R _ C A _ R 1 7 _ I n t r a - C o r e | R4 | RP-211757 | Ericsson |
| 881205 | Perf. Part: NR_CA_R17_Intra | N R _ C A _ R 1 7 _ I n t r a - P e r f | R4 | RP-211757 | Ericsson |
| 0 | Rel-17 NR inter band Carrier Aggregation (iterbCA) | - |  | - | - |
| 900063 | High power UE for NR TDD intra-band Carrier Aggregation in frequency range FR1 | N R _ i n t r a _ H P U E _ R 1 7 |  | RP-212180 | Huawei |
| 900163 | Core part: NR_intra_HPUE_R17 | N R _ i n t r a _ H P U E _ R 1 7 - C o r e | R4 | RP-212180 | Huawei |
| 900263 | Perf. Part: NR_intra_HPUE_R17 | N R _ i n t r a _ H P U E _ R 1 7 - P e r f | R4 | RP-212180 | Huawei |
| 881006 | NR iterbCA/Dual Connectivity for 2 bands DL with x bands UL (x=1,2) | N R _ C A D C _ R 1 7 _ 2 B D L _ x B U L |  | RP-212511 | ZTE |
| 881106 | Core part: NR_CADC_R17_2BDL_xBUL | N R _ C A D C _ R 1 7 _ 2 B D L _ x B U L - C o r e | R4 | RP-212511 | ZTE |
| 881206 | Perf. part: NR_CADC_R17_2BDL_xBUL | N R _ C A D C _ R 1 7 _ 2 B D L _ x B U L - P e r f | R4 | RP-212511 | ZTE |
| 881007 | NR iterbCA for 3 bands DL with 1 band UL | N R _ C A _ R 1 7 _ 3 B D L _ 1 B U L |  | RP-212239 | CATT |
| 881107 | Core part: NR_CA_R17_3BDL_1BUL | N R _ C A _ R 1 7 _ 3 B D L _ 1 B U L - C o r e | R4 | RP-212239 | CATT |
| 881207 | Perf. part: NR_CA_R17_3BDL_1BUL | N R _ C A _ R 1 7 _ 3 B D L _ 1 B U L - P e r f | R4 | RP-212239 | CATT |
| 881008 | NR iterbCA/Dual Connectivity for 3 bands DL with 2 bands UL | N R _ C A D C _ R 1 7 _ 3 B D L _ 2 B U L |  | RP-212512 | ZTE |
| 881108 | Core part: NR_CADC_R17_3BDL_2BUL | N R _ C A D C _ R 1 7 _ 3 B D L _ 2 B U L - C o r e | R4 | RP-212512 | ZTE |
| 881208 | Perf. part: NR_CADC_R17_3BDL_2BUL | N R _ C A D C _ R 1 7 _ 3 B D L _ 2 B U L - P e r f | R4 | RP-212512 | ZTE |
| 881009 | NR iterbCA for 4 bands DL with 1 band UL | N R _ C A _ R 1 7 _ 4 B D L _ 1 B U L |  | RP-211759 | Ericsson |
| 881109 | Core part: NR_CA_R17_4BDL_1BUL | N R _ C A _ R 1 7 _ 4 B D L _ 1 B U L - C o r e | R4 | RP-211759 | Ericsson |
| 881209 | Perf. part: NR_CA_R17_4BDL_1BUL | N R _ C A _ R 1 7 _ 4 B D L _ 1 B U L - P e r f | R4 | RP-211759 | Ericsson |
| 881010 | NR iterbCA/Dual connectivity for DL 4 bands and 2UL bands | N R _ C A D C _ R 1 7 _ 4 B D L _ 2 B U L |  | RP-212110 | Samsung |
| 881110 | Core part: NR_CADC_R17_4BDL_2BUL | N R _ C A D C _ R 1 7 _ 4 B D L _ 2 B U L - C o r e | R4 | RP-212110 | Samsung |
| 881210 | Perf. part: NR_CADC_R17_4BDL_2BUL | N R _ C A D C _ R 1 7 _ 4 B D L _ 2 B U L - P e r f | R4 | RP-212110 | Samsung |
| 881011 | NR iterbCA for 5 bands DL with x bands UL (x=1, 2) | N R _ C A D C _ R 1 7 _ 5 B D L _ x B U L |  | RP-212176 | Huawei |
| 881111 | Core part: NR_CADC_R17_5BDL_xBUL | N R _ C A D C _ R 1 7 _ 5 B D L _ x B U L - C o r e | R4 | RP-212176 | Huawei |
| 881211 | Perf. part: NR_CADC_R17_5BDL_xBUL | N R _ C A D C _ R 1 7 _ 5 B D L _ x B U L - P e r f | R4 | RP-212176 | Huawei |
| 890054 | Rel-17 High power UE for NR inter-band Carrier Aggregation with 2 bands downlink and x bands uplink (x=1,2) | N R _ P C 2 _ C A _ R 1 7 _ 2 B D L _ 2 B U L |  | RP-211831 | China Telecom |
| 890154 | Core part: NR_PC2_CA_R17_2BDL_2BUL | N R _ P C 2 _ C A _ R 1 7 _ 2 B D L _ 2 B U L - C o r e | R4 | RP-211831 | China Telecom |
| 890254 | Perf. part: NR_PC2_CA_R17_2BDL_2BUL | N R _ P C 2 _ C A _ R 1 7 _ 2 B D L _ 2 B U L - P e r f | R4 | RP-211831 | China Telecom |
| 920066 | UE Conformance - Rel-17 High power UE for NR inter-band Carrier Aggregation with 2 bands downlink and x bands uplink (x=1,2) | N R _ P C 2 _ C A _ R 1 7 _ 2 B D L _ 2 B U L - U E C o n T e s t | R5 | RP-211140 | China Telecom |
| 0 | Rel-17 NR and LTE | - |  | - | - |
| 920072 | LTE/NR spectrum sharing in Band 34/n34 and Band 39/n39 | D S S _ L T E _ B 3 4 _ N R _ B n 3 4 _ L T E _ B 3 9 _ N R _ B n 3 9 | R4 |  | CMCC |
| 920172 | Core part: LTE/NR spectrum sharing in Band 34/n34 and Band 39/n39 | D S S _ L T E _ B 3 4 _ N R _ B n 3 4 _ L T E _ B 3 9 _ N R _ B n 3 9 - C o r e | R4 | RP-211996 | CMCC |
| 920073 | High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands | L T E _ N R _ H P U E _ F W V M | R4 | RP-212533 | Nokia |
| 920173 | Core part: High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands | L T E _ N R _ H P U E _ F W V M - C o r e | R4 | RP-212533 | Nokia |
| 920273 | Perf. part: High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands | L T E _ N R _ H P U E _ F W V M - P e r f | R4 | RP-212533 | Nokia |
| 920074 | Rel-17 Dual Connectivity (DC) of x bands (x=1,2) LTE inter-band CA (xDL1UL) and 4 bands NR inter-band CA (4DL1UL) | D C _ R 1 7 _ x B L T E _ 4 B N R _ y D L 2 U L | R4 | RP-212606 | Huawei |
| 920174 | Core part: Rel-17 Dual Connectivity (DC) of x bands (x=1,2) LTE inter-band CA (xDL1UL) and 4 bands NR inter-band CA (4DL1UL) | D C _ R 1 7 _ x B L T E _ 4 B N R _ y D L 2 U L - C o r e | R4 | RP-212606 | Huawei |
| 920274 | Perf. part: Rel-17 Dual Connectivity (DC) of x bands (x=1,2) LTE inter-band CA (xDL1UL) and 4 bands NR inter-band CA (4DL1UL) | D C _ R 1 7 _ x B L T E _ 4 B N R _ y D L 2 U L - P e r f | R4 | RP-212606 | Huawei |
| 911019 | Addition of MSD (Maximum Sensitivity Degradation) for inter-band EN-DC combinations (1 band LTE+1 band NR FR1) due to added channel bandwidths | E N D C _ R 1 7 _ 1 B L T E _ 1 B N R _ M S |  | RP-212187 | Huawei, HiSilicon |
| 911119 | Core part: ENDC_R17_1BLTE_1BNR_MS | E N D C _ R 1 7 _ 1 B L T E _ 1 B N R _ M S D - C o r e |  | RP-212187 | Huawei, HiSilicon |
| 911011 | Rel-17 Power Class 2 for EN-DC with x LTE bands + y NR band(s) in DL and with 1 LTE band +1 TDD NR band in UL (either x= 2, 3, y=1 or x=1, 2, y=2) | E N D C _ P C 2 _ R 1 7 _ x L T E _ y N R |  | RP-211760 | Ericsson |
| 911111 | Core part: ENDC_PC2_R17_xLTE_yNR | E N D C _ P C 2 _ R 1 7 _ x L T E _ y N R - C o r e | R4 | RP-211760 | Ericsson |
| 911211 | Perf. Part: ENDC_PC2_R17_xLTE_yNR | E N D C _ P C 2 _ R 1 7 _ x L T E _ y N R - P e r f | R4 | RP-211760 | Ericsson |
| 930051 | UE Conformance - Power Class 2 for EN-DC with x LTE bands + y NR band(s) in DL and with 1 LTE band +1 TDD NR band in UL (either x= 2, 3, y=1 or x=1, 2, y=2) | E N D C _ P C 2 _ R 1 7 _ x L T E _ y N R - U E C o n T e s t | R5 | RP-211870 | Ericsson |
| 911018 | Simultaneous Rx/Tx band combinations for NR CA/DC, NR SUL and LTE/NR DC | L T E _ N R _ S i m u l t _ R x T x |  | RP-211382 | Huawei |
| 911118 | Core part: LTE_NR_Simult_RxTx | L T E _ N R _ S i m u l t _ R x T x - C o r e | R4 | RP-211382 | Huawei |
| 911218 | Perf. part: LTE_NR_Simult_RxTx | L T E _ N R _ S i m u l t _ R x T x - P e r f | R4 | RP-211382 | Huawei |
| 890052 | Rel-17 Dual Connectivity (DC) of 5 bands LTE inter-band CA (5DL/1UL) and 1 NR band (1DL/1UL) | D C _ R 1 7 _ 5 B L T E _ 1 B N R _ 6 D L 2 U L |  | RP-211195 | Samsung |
| 890152 | Core part: DC_R17_5BLTE_1BNR_6DL2UL | D C _ R 1 7 _ 5 B L T E _ 1 B N R _ 6 D L 2 U L - C o r e | R4 | RP-211195 | Samsung |
| 890252 | Perf. part: DC_R17_5BLTE_1BNR_6DL2UL | D C _ R 1 7 _ 5 B L T E _ 1 B N R _ 6 D L 2 U L - P e r f | R4 | RP-211195 | Samsung |
| 890053 | Rel-17 Dual Connectivity (DC) of x bands (x=2, 3, 4) LTE inter-band CA (xDL/1UL) and 1 NR FR1 band (1DL/1UL) and 1 NR FR2 band (1DL/1UL) | D C _ R 1 7 _ x B L T E _ 2 B N R _ y D L 3 U L |  | RP-202542 | Samsung |
| 890153 | Core part: DC_R17_xBLTE_2BNR_yDL3UL | D C _ R 1 7 _ x B L T E _ 2 B N R _ y D L 3 U L - C o r e | R4 | RP-202542 | Samsung |
| 890253 | Perf. part: DC_R17_xBLTE_2BNR_yDL3UL | D C _ R 1 7 _ x B L T E _ 2 B N R _ y D L 3 U L - P e r f | R4 | RP-202542 | Samsung |
| 880080 | Band combinations for concurrent operation of NR/LTE Uu bands/band combinations and one NR/LTE V2X PC5 band | N R _ L T E _ V 2 X _ P C 5 _ c o m b o s |  | RP-210588 | CATT |
| 880180 | Core part: NR_LTE_V2X_PC5_combos | N R _ L T E _ V 2 X _ P C 5 _ c o m b o s - C o r e | R4 | RP-210588 | CATT |
| 880280 | Perf. part: NR_LTE_V2X_PC5_combos | N R _ L T E _ V 2 X _ P C 5 _ c o m b o s - P e r f | R4 | RP-210588 | CATT |
| 880094 | High power UE (power class 2) for EN-DC with 1 LTE band + 1 NR TDD band | E N D C _ U E _ P C 2 _ R 1 7 _ N R _ T D D |  | RP-211853 | China Unicom |
| 880194 | Core part: ENDC_UE_PC2_R17_NR_TDD | E N D C _ U E _ P C 2 _ R 1 7 _ N R _ T D D - C o r e | R4 | RP-211853 | China Unicom |
| 880294 | Perf. part: ENDC_UE_PC2_R17_NR_TDD | E N D C _ U E _ P C 2 _ R 1 7 _ N R _ T D D - P e r f | R4 | RP-211853 | China Unicom |
| 911000 | UE Conformance - ENDC_UE_PC2_R17_NR_TDD | E N D C _ U E _ P C 2 _ R 1 7 _ N R _ T D D - U E C o n T e s t | R5 | RP-211420 | China Unicom |
| 880095 | LTE/NR spectrum sharing in Band 38/n38 | D S S _ L T E _ B 3 8 _ N R _ B n 3 8 |  | RP-201314 | Vodafone |
| 880195 | Core part: DSS_LTE_B38_NR_Bn38 | D S S _ L T E _ B 3 8 _ N R _ B n 3 8 - C o r e | R4 | RP-201314 | Vodafone |
| 880096 | LTE/NR spectrum sharing in Band 40/n40 | D S S _ L T E _ B 4 0 _ N R _ B n 4 0 |  | RP-202084 | Reliance Jio |
| 880196 | Core part: DSS_LTE_B40_NR_Bn40 | D S S _ L T E _ B 4 0 _ N R _ B n 4 0 - C o r e | R4 | RP-202084 | Reliance Jio |
| 880296 | Perf. part: DSS_LTE_B40_NR_Bn40 | D S S _ L T E _ B 4 0 _ N R _ B n 4 0 - P e r f | R4 | RP-202084 | Reliance Jio |
| 880098 | Dual Connectivity (DC) of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL) | D C _ R 1 7 _ 1 B L T E _ 1 B N R _ 2 D L 2 U L |  | RP-212096 | CHTTL |
| 880198 | Core part: DC_R17_1BLTE_1BNR_2DL2UL | D C _ R 1 7 _ 1 B L T E _ 1 B N R _ 2 D L 2 U L - C o r e | R4 | RP-212096 | CHTTL |
| 880298 | Perf. part: DC_R17_1BLTE_1BNR_2DL2UL | D C _ R 1 7 _ 1 B L T E _ 1 B N R _ 2 D L 2 U L - P e r f | R4 | RP-212096 | CHTTL |
| 880099 | Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL) | D C _ R 1 7 _ 2 B L T E _ 1 B N R _ 3 D L 2 U L |  | RP-212174 | Huawei |
| 880199 | Core part: DC_R17_2BLTE_1BNR_3DL2UL | D C _ R 1 7 _ 2 B L T E _ 1 B N R _ 3 D L 2 U L - C o r e | R4 | RP-212174 | Huawei |
| 890263 | Perf. part: DC_R17_2BLTE_1BNR_3DL2UL | D C _ R 1 7 _ 2 B L T E _ 1 B N R _ 3 D L 2 U L - P e r f | R4 | RP-212174 | Huawei |
| 881000 | Dual Connectivity (DC) of 3 bands LTE inter-band CA (3DL/1UL) and 1 NR band (1DL/1UL) | D C _ R 1 7 _ 3 B L T E _ 1 B N R _ 4 D L 2 U L |  | RP-211758 | Ericsson |
| 881100 | Core part: DC_R17_3BLTE_1BNR_4DL2UL | D C _ R 1 7 _ 3 B L T E _ 1 B N R _ 4 D L 2 U L - C o r e | R4 | RP-211758 | Ericsson |
| 881200 | Perf. part: DC_R17_3BLTE_1BNR_4DL2UL | D C _ R 1 7 _ 3 B L T E _ 1 B N R _ 4 D L 2 U L - P e r f | R4 | RP-211758 | Ericsson |
| 881001 | Dual Connectivity (DC) of 4 bands LTE inter-band CA (4DL/1UL) and 1 NR band (1DL/1UL) | D C _ R 1 7 _ 4 B L T E _ 1 B N R _ 5 D L 2 U L |  | RP-212532 | Nokia |
| 881101 | Core part: DC_R17_4BLTE_1BNR_5DL2UL | D C _ R 1 7 _ 4 B L T E _ 1 B N R _ 5 D L 2 U L - C o r e | R4 | RP-212532 | Nokia |
| 881201 | Perf. part: DC_R17_4BLTE_1BNR_5DL2UL | D C _ R 1 7 _ 4 B L T E _ 1 B N R _ 5 D L 2 U L - P e r f | R4 | RP-212532 | Nokia |
| 881002 | Dual Connectivity (DC) of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) | D C _ R 1 7 _ x B L T E _ 2 B N R _ y D L 2 U L |  | RP-211750 | LG Electronics |
| 881102 | Core part: DC_R17_xBLTE_2BNR_yDL2UL | D C _ R 1 7 _ x B L T E _ 2 B N R _ y D L 2 U L - C o r e | R4 | RP-211750 | LG Electronics |
| 881202 | Perf. part: DC_R17_xBLTE_2BNR_yDL2UL | D C _ R 1 7 _ x B L T E _ 2 B N R _ y D L 2 U L - P e r f | R4 | RP-211750 | LG Electronics |
| 881003 | Dual Connectivity (DC) of x bands (x=1,2) LTE inter-band CA (xDL/xUL) and y bands (y=3-x) NR inter-band CA (yDL/yUL) | D C _ R 1 7 _ x B L T E _ y B N R _ 3 D L 3 U L |  | RP-211803 | ZTE |
| 881103 | Core part: DC_R17_xBLTE_yBNR_3DL3UL | D C _ R 1 7 _ x B L T E _ y B N R _ 3 D L 3 U L - C o r e | R4 | RP-211803 | ZTE |
| 881203 | Perf. part: DC_R17_xBLTE_yBNR_3DL3UL | D C _ R 1 7 _ x B L T E _ y B N R _ 3 D L 3 U L - P e r f | R4 | RP-211803 | ZTE |
| 881004 | Dual Connectivity (DC) of x bands (x=1,2,3) LTE inter-band CA (xDL/1UL) and 3 bands NR inter-band CA (3DL/1UL) | D C _ R 1 7 _ x B L T E _ 3 B N R _ y D L 2 U L |  | RP-211805 | ZTE |
| 881104 | Core part: DC_R17_xBLTE_3BNR_yDL2UL | D C _ R 1 7 _ x B L T E _ 3 B N R _ y D L 2 U L - C o r e | R4 | RP-211805 | ZTE |
| 881204 | Perf. part: DC_R17_xBLTE_3BNR_yDL2UL | D C _ R 1 7 _ x B L T E _ 3 B N R _ y D L 2 U L - P e r f | R4 | RP-211805 | ZTE |
| 881012 | band combinations for SA NR supplementary uplink (SUL), NSA NR SUL, NSA NR SUL with UL sharing from the UE perspective (ULSUP) | N R _ S U L _ c o m b o s _ R 1 7 |  | RP-212178 | Huawei |
| 881112 | Core part: NR_SUL_combos_R17 | N R _ S U L _ c o m b o s _ R 1 7 - C o r e | R4 | RP-212178 | Huawei |
| 881212 | Perf. part: NR_SUL_combos_R17 | N R _ S U L _ c o m b o s _ R 1 7 - P e r f | R4 | RP-212178 | Huawei |
| 900056 | UE Conformance - Rel-17 NR CA and DC; and NR and LTE DC Configurations | N R _ C A D C _ N R _ L T E _ D C _ R 1 7 - U E C o n T e s t | R5 | RP-211357 | Huawei |
| 0 | Rel-17 NR new/modified bands | - |  | - | - |
| 911016 | Introduction of 900MHz NR band for Europe for Rail Mobile Radio (RMR) | N R _ R A I L _ E U _ 9 0 0 M H z |  | RP-211495 | UIC |
| 911116 | Core part: NR_RAIL_EU_900MHz | N R _ R A I L _ E U _ 9 0 0 M H z - C o r e | R4 | RP-211495 | UIC |
| 911216 | Perf. part: NR_RAIL_EU_900MHz | N R _ R A I L _ E U _ 9 0 0 M H z - P e r f | R4 | RP-211495 | UIC |
| 911017 | Introduction of 1900MHz NR TDD band for Europe for Rail Mobile Radio (RMR) | N R _ R A I L _ E U _ 1 9 0 0 M H z _ T D D |  | RP-211542 | UIC |
| 911117 | Core part: NR_RAIL_EU_1900MHz_TDD | N R _ R A I L _ E U _ 1 9 0 0 M H z _ T D D - C o r e | R4 | RP-211542 | UIC |
| 911217 | Perf. part: NR_RAIL_EU_1900MHz_TDD | N R _ R A I L _ E U _ 1 9 0 0 M H z _ T D D - P e r f | R4 | RP-211542 | UIC |
| 860041 | Extending current NR operation to 71GHz | N R _ e x t _ t o _ 7 1 G H z | R1 | RP-212637 | Qualcomm |
| 860141 | Core part: Extending current NR operation to 71GHz | N R _ e x t _ t o _ 7 1 G H z - C o r e | R1 | RP-212637 | Qualcomm |
| 860241 | Perf. part: Extending current NR operation to 71GHz | N R _ e x t _ t o _ 7 1 G H z - P e r f | R4 | RP-212637 | Qualcomm |
| 860037 | Study on supporting NR from 52.6 GHz to 71 GHz | F S _ N R _ 5 2 _ t o _ 7 1 G H z | R1 | RP-201838 | Intel |
| 870060 | Study on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz | F S _ N R _ I M T _ p a r a m | R4 | RP-202276 | Ericsson |
| 870063 | Introduction of NR band n13 | N R _ n 1 3 | R4 | RP-200480 | Huawei |
| 870163 | Core part: Introduction of NR band n13 | N R _ n 1 3 - C o r e | R4 | RP-200480 | Huawei |
| 870263 | Perf. part: Introduction of NR band n13 | N R _ n 1 3 - P e r f | R4 | RP-200480 | Huawei |
| 880082 | Introduction of NR 47GHz band | N R _ 4 7 G H z _ b a n d | R4 | RP-212528 | Ericsson |
| 880182 | Core part: Introduction of NR 47GHz band | N R _ 4 7 G H z _ b a n d - C o r e | R4 | RP-212528 | Ericsson |
| 880282 | Perf. part: Introduction of NR 47GHz band | N R _ 4 7 G H z _ b a n d - P e r f | R4 | RP-212528 | Ericsson |
| 880084 | Introduction of NR band 24 | N R _ b a n d _ n 2 4 | R4 | RP-211505 | Ligado Networks |
| 880184 | Core part: Introduction of NR band 24 | N R _ b a n d _ n 2 4 - C o r e | R4 | RP-211505 | Ligado Networks |
| 880284 | Perf. part: Introduction of NR band 24 | N R _ b a n d _ n 2 4 - P e r f | R4 | RP-211505 | Ligado Networks |
| 890050 | Introduction of 6GHz NR licensed bands | N R _ 6 G H z | R4 | RP-202844 | Huawei |
| 890150 | Core part: Introduction of 6GHz NR licensed bands | N R _ 6 G H z - C o r e | R4 | RP-202844 | Huawei |
| 890250 | Perf. part: Introduction of 6GHz NR licensed bands | N R _ 6 G H z - P e r f | R4 | RP-202844 | Huawei |
| 920075 | Introduction of operation in full unlicensed band 5925-7125MHz for NR | N R _ 6 G H z _ u n l i c _ f u l l | R4 | RP-212302 | Apple |
| 920175 | Core part: Introduction of operation in full unlicensed band 5925-7125MHz for NR | N R _ 6 G H z _ u n l i c _ f u l l - C o r e | R4 | RP-212302 | Apple |
| 920275 | Perf. part: Introduction of operation in full unlicensed band 5925-7125MHz for NR | N R _ 6 G H z _ u n l i c _ f u l l - P e r f | R4 | RP-212302 | Apple |
| 890051 | Introduction of lower 6GHz NR unlicensed operation for Europe | N R _ 6 G H z _ u n l i c _ E U | R4 | RP-212625 | Nokia |
| 890151 | Core part: Introduction of lower 6GHz NR unlicensed operation for Europe | N R _ 6 G H z _ u n l i c _ E U - C o r e | R4 | RP-212625 | Nokia |
| 890251 | Perf. part: Introduction of lower 6GHz NR unlicensed operation for Europe | N R _ 6 G H z _ u n l i c _ E U - P e r f | R4 | RP-212625 | Nokia |
| 900065 | Introduction of NR band n67 | N R _ n 6 7 | R4 | RP-202829 | Ericsson |
| 900165 | Core part: Introduction of NR band n67 | N R _ n 6 7 - C o r e | R4 | RP-202829 | Ericsson |
| 900265 | Perf. Part: Introduction of NR band n67 | N R _ n 6 7 - P e r f | R4 | RP-202829 | Ericsson |
| 900066 | Introduction of NR band n85 | N R _ n 8 5 | R4 | RP-210707 | Ericsson |
| 900166 | Core part: Introduction of NR band n85 | N R _ n 8 5 - C o r e | R4 | RP-210707 | Ericsson |
| 900266 | Perf. part: Introduction of NR band n85 | N R _ n 8 5 - P e r f | R4 | RP-210707 | Ericsson |
| 880083 | Introduction of 1.6 GHz NR supplemental uplink (SUL) band with same uplink frequency range of Band 24 | N R _ S U L _ U L _ n 2 4 |  | RP-210341 | Ligado Networks |
| 880183 | Core part: NR_SUL_UL_n24 | N R _ S U L _ U L _ n 2 4 - C o r e | R4 | RP-210341 | Ligado Networks |
| 880283 | Perf. part: NR_SUL_UL_n24 | N R _ S U L _ U L _ n 2 4 - P e r f | R4 | RP-210341 | Ligado Networks |
| 880085 | Introduction of NR supplemental uplink (SUL) band 1880-1920MHz | N R _ S U L _ b a n d _ 1 8 8 0 _ 1 9 2 0 M H z |  | RP-201363 | CMCC |
| 880185 | Core part: NR_SUL_band_1880_1920MHz | N R _ S U L _ b a n d _ 1 8 8 0 _ 1 9 2 0 M H z - C o r e | R4 | RP-201363 | CMCC |
| 880285 | Perf. part: NR_SUL_band_1880_1920MHz | N R _ S U L _ b a n d _ 1 8 8 0 _ 1 9 2 0 M H z - P e r f | R4 | RP-201363 | CMCC |
| 880086 | Introduction of NR supplemental uplink (SUL) band 2300-2400MHz | N R _ S U L _ b a n d _ 2 3 0 0 _ 2 4 0 0 M H z |  | RP-201364 | CMCC |
| 880186 | Core part: NR_SUL_band_2300_2400MHz | N R _ S U L _ b a n d _ 2 3 0 0 _ 2 4 0 0 M H z - C o r e | R4 | RP-201364 | CMCC |
| 880286 | Perf. part: NR_SUL_band_2300_2400MHz | N R _ S U L _ b a n d _ 2 3 0 0 _ 2 4 0 0 M H z - P e r f | R4 | RP-201364 | CMCC |
| 900055 | UE Conformance - New Rel-17 NR licensed bands and extension of existing NR bands | N R _ l i c _ b a n d s _ B W _ R 1 7 - U E C o n T e s t | R5 | RP-202567 | Huawei |
| 0 | Rel-17 LTE new/modified bands | - |  | - | - |
| 911020 | New bands and bandwidth allocation for 5G terrestrial broadcast - part 1 | L T E _ t e r r _ b c a s t _ b a n d s _ p a r t 1 | R1 | RP-211144 | EBU |
| 911120 | New bands and bandwidth allocation for 5G terrestrial broadcast - part 1 | L T E _ t e r r _ b c a s t _ b a n d s _ p a r t 1 | R1 | RP-211144 | EBU |
| 880079 | Additional LTE bands for UE categories M1/M2/NB1/NB2 | L T E _ b a n d s _ R 1 7 _ M 1 _ M 2 _ N B 1 _ N B 2 |  | RP-211857 | Ericsson |
| 880179 | Core part: LTE_bands_R17_M1_M2_NB1_NB2 | L T E _ b a n d s _ R 1 7 _ M 1 _ M 2 _ N B 1 _ N B 2 - C o r e | R4 | RP-211857 | Ericsson |
| 880279 | Perf. part: LTE_bands_R17_M1_M2_NB1_NB2 | L T E _ b a n d s _ R 1 7 _ M 1 _ M 2 _ N B 1 _ N B 2 - P e r f | R4 | RP-211857 | Ericsson |
| 880081 | Modification of LTE Band 24 Specifications to comply with updated regulatory emission limits | L T E _ B 2 4 _ m o d |  | RP-211507 | Ligado Networks |
| 880181 | Core part: LTE_B24_mod | L T E _ B 2 4 _ m o d - C o r e | R4 | RP-211507 | Ligado Networks |
| 880281 | Perf. part: LTE_B24_mod | L T E _ B 2 4 _ m o d - P e r f | R4 | RP-211507 | Ligado Networks |
| 920067 | UE Conformance - Modification of LTE Band 24 Specifications to comply with updated regulatory emission limits | L T E _ B 2 4 _ m o d - U E C o n T e s t | R5 | RP-212497 | Ligado Networks |
| 0 | Rel-17 LTE inter-band Carrier Aggregation (LTE iterbCA) | - |  | - | - |
| 880087 | LTE iterbCA for 2 bands DL with 1 band UL | L T E _ C A _ R 1 7 _ 2 B D L _ 1 B U L |  | RP-211859 | Qualcomm |
| 880187 | Core part: LTE_CA_R17_2BDL_1BUL | L T E _ C A _ R 1 7 _ 2 B D L _ 1 B U L - C o r e | R4 | RP-211859 | Qualcomm |
| 880287 | Perf. part: LTE_CA_R17_2BDL_1BUL | L T E _ C A _ R 1 7 _ 2 B D L _ 1 B U L - P e r f | R4 | RP-211859 | Qualcomm |
| 880088 | LTE iterbCA for 3 bands DL with 1 band UL | L T E _ C A _ R 1 7 _ 3 B D L _ 1 B U L |  | RP-212192 | Huawei |
| 880188 | Core part: LTE_CA_R17_3BDL_1BUL | L T E _ C A _ R 1 7 _ 3 B D L _ 1 B U L - C o r e | R4 | RP-212192 | Huawei |
| 880288 | Stopped - Perf. part: LTE iterbCA for 3 bands DL with 1 band UL | L T E _ C A _ R 1 7 _ 3 B D L _ 1 B U L - P e r f | R4 | RP-202615 | Huawei |
| 880089 | LTE iterbCA Core for x bands DL (x=4, 5, 6) with 1 band UL | L T E _ C A _ R 1 7 _ x B D L _ 1 B U L |  | RP-211885 | Nokia |
| 880189 | Core part: LTE_CA_R17_xBDL_1BUL | L T E _ C A _ R 1 7 _ x B D L _ 1 B U L - C o r e | R4 | RP-211885 | Nokia |
| 880289 | Perf. part: LTE_CA_R17_xBDL_1BUL | L T E _ C A _ R 1 7 _ x B D L _ 1 B U L - P e r f | R4 | RP-211885 | Nokia |
| 880090 | LTE iterbCA for 2 bands DL with 2 bands UL | L T E _ C A _ R 1 7 _ 2 B D L _ 2 B U L |  | RP-212194 | Huawei |
| 880190 | Core part: LTE_CA_R17_2BDL_2BUL | L T E _ C A _ R 1 7 _ 2 B D L _ 2 B U L - C o r e | R4 | RP-212194 | Huawei |
| 880290 | Perf. part: LTE_CA_R17_2BDL_2BUL | L T E _ C A _ R 1 7 _ 2 B D L _ 2 B U L - P e r f | R4 | RP-212194 | Huawei |
| 880091 | LTE iterbCA for x bands DL (x= 3, 4, 5) with 2 bands UL | L T E _ C A _ R 1 7 _ x B D L _ 2 B U L |  | RP-211748 | LG |
| 880191 | Core part: LTE_CA_R17_xBDL_2BUL | L T E _ C A _ R 1 7 _ x B D L _ 2 B U L - C o r e | R4 | RP-211748 | LG |
| 880291 | Perf. part: LTE_CA_R17_xBDL_2BUL | L T E _ C A _ R 1 7 _ x B D L _ 2 B U L - P e r f | R4 | RP-211748 | LG |

Annex B:
Template for contributing

<guidance: Copy all this section into a tdoc and use it as a template, then update, then submit as a tdoc to a plenary.
This document is the template for a contribution to TR 21.917 ("Release 17 Description; Summary of Rel-17 Work Items"). It is meant to be endorsed by the plenary.

<Overall guidance:

When completed, remove all the red guidance text.

Each summary is expected to be one page long maximum.

The target audience is non-specialist engineers inside and outside the 3GPP community. The terminology and abbreviations have to be understandable by any engineer, or have to be explained.

The summary should reflect the actual outcome of the Feature - and not the initial intentions.

Once written, it is advised to ask a colleague who was not involved in this topic to read this summary and check her/his understanding.

>

Summary based on the input provided by [company name] in [tdoc number as per the top line].

Copy/paste from the attached Excel table all the lines covered by this summary.

Example table (to be updated):

| Unique ID | Name | Acronym | WG | WID | WI Rapporteur name |
| --- | --- | --- | --- | --- | --- |
| 910065 | Enhanced support of Non-Public Networks | eNPN | S2 | SP-200980 |  |
| 900015 | Stage 2 for eNPN | eNPN | S2 | SP-200980 |  |
| 910016 | CT aspects of eNPN | eNPN | Ct | CP-212103 |  |
| 910066 | CT1 aspects of eNPN | eNPN | C1 | CP-212103 |  |
| 910067 | CT3 aspects of eNPN | eNPN | C3 | CP-212103 |  |
| 910068 | CT4 aspects of eNPN | eNPN | C4 | CP-212103 |  |
| 920025 | Security Aspects of eNPN | eNPN | S3 | SP-210422 |  |
| 880008 | Study on enhanced security support for Non-Public Networks | FS_eNPN_SEC | S3 | SP-200353 |  |

Introduction

Your introduction text here.

<guidance:
- summarize in one to a few sentences what was achieved by this WI and how: what problems it solves, what new functionality it adds, etc.;

- clarify what is the relation of this WI to other WIs (e.g. is this an enhancement of a former WI? Is there a related WI?) and what was the status before this WI

Description

Your description text here.

<guidance:

- Explain here in a couple of paragraphs the main impacts on the system, mostly: the new and/or impacted entities, and the big lines of the system's behaviour;

- cover all aspects: Stage 1, Stage 2, and what is relevant of Stage 3, charging, security, testing, etc

- When the usual Stage 1/Stage 2/Stage 3 process is not followed -and in particular if a full stage is missing- explain why.

- this it not intended to be a copy and paste from the specifications, the WID or status report;

- the reader shall understand how the key functionalities are working without running too far into details;

- use figures, tables, references to illustrate and  facilitate the understanding;

>

References

<guidance: add some references where the interested reader can find further documentation, e.g.

- new/impacted TRs and TSs (if any)

- last RAN status report (if any)

-NO NEED to reference the WID, since it is already shown in the table above

- the list of CRs relating to this Feature will be automatically generated by the 3GPP portal by replacing the UID tag(s) by their actual value in the line

https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=[UID1],[UID2 if any],[UID3 if any],[etc]

e.g.:

https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=830080,810060,800098,830180

.>

List of related CRs: select "TSG Status = Approved" in: 
https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=[UID1],[UID2 if any],[UID3 if any],[etc]

[1]	Reference 1, e.g. TR 38.825: “Study on NR industrial Internet of Things (IoT)”

[2]	Reference 2, e.g. TS 23.501: “System architecture for the 5G System (5GS)”

Annex C:
Process to get further information

# C.1	General

Since the present document is limited to provide an overview of each Feature, this chapter explains how to get additional information, in particular how to retrieve all the Specifications (TSs) and Reports (TRs) as well as all the CRs which relate to a given Work Item.

The Unique Identifier (UID) is the key to get additional information on a given Work Item. It can be found in the table located just below the clause's header. The table has the following format:

Table B-1: table format

| Unique Identifier (UID) | Name | Acronym | Outline Level (1=Feature, 2=Building Block, 3=Work Task) | Responsible Working Group | Work Item Description |
| --- | --- | --- | --- | --- | --- |

For readability reasons, the table headers are omitted in the continuation of the present document.

For instance, for the "Mission Critical Push to Talk over LTE Realignment", the table has to be understood as:

Table B-2: Example of table at the introduction of each Feature

| Unique Identifier (UID) | Name | Acronym | Outline Level | Responsible Working Group | Work Item Description |
| --- | --- | --- | --- | --- | --- |
| 700029 | Mission Critical Push to Talk over LTE Realignment | MCImp-MCPTTR | 2 | S1 | SP-150821 |

Thus, the UID for this Work Item is 700029.

Two methods are now possible to retrieve more information on a given feature: the "Step by step method" and the "Direct method". The "direct method" is faster but implies to know the hierarchical structure of the Work Items. The "step by step method" is slower but is easier to use, in particular when the hierarchical structure is unknown.

For instance, for retrieving all the CRs that relate to "Enhancements for Mission Critical Push To Talk", the search has to be done on UID 740022 but also potentially on its children Work Items (UID 720056, 740023 and 740024).

Table B-3: Example of a hierarchical structure and its consequences on the search procedure

| 740022 | Enhancements for Mission Critical Push To Talk | MCImp-eMCPTT |  | SP-160490 |
| --- | --- | --- | --- | --- |
| 720056 | Stage 2 of Enhancements for Mission Critical Push To Talk | MCImp-eMCPTT | S6 | SP-160490 |
| 740023 | Stage 3 of Enhancements for Mission Critical Push To Talk | MCImp-eMCPTT-CT | CT | CP-160824 |
| 740024 | CT1 aspects of Enhancements for Mission Critical Push To Talk | MCImp-eMCPTT-CT | C1 | CP-160824 |

These two methods are described in the following clauses.

# C.2	Direct method

The links below lead to the pages containing respectively all the Specifications and all the Change Requests (CRs) linked to a given Feature:

https://portal.3gpp.org/Specifications.aspx?q=1&WiUid=[UID]

https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=[UID]

where "[UID]" has to be preplaced by the UID value.

Using the example provided in the table 4.1-3, the specification linked to "Stage 2 of Enhancements for Mission Critical Push To Talk" can be found in:

https://portal.3gpp.org/Specifications.aspx?q=1&WiUid=720056

And all the related Change Requests are listed in:

https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=720056

# C.3	Step by step method

This method is to be used when the hierarchical structure is not known or when the "direct method" above does not show the expected results.

In this case, the 3GPP Ultimate web site has to be used:

https://portal.3gpp.org

As a preliminary step, it is essential that the "Customized Selection" is set to "All TSGs" (otherwise, a filter would be applied).

Figure C.3-1: Selecting "All TSGs" in "Customized Selection" as to remove any potential filter on the Search

Then select the "Work Plan" tab (upper red arrow in the figure below).

Figure C.3-2: Using the 3GU Portal to retrieve more information on a given Feature

Then the search might be performed by either typing the Acronym (as shown by the left red arrow on the figure above, using the example "MCPTT"), or by the name or UID (right box) then by clicking on the "Search" button. Watch the "Granularity (Level)" field, which is a filter to return only the Items which level is specified here.

In the results, the icon depicting some binoculars has to be hit (lower right red arrow on the figure above).

This will lead to the page shown in the figure below:

Figure C.3-3: Window resulting from a "Search"

On this window, the "Related" tab has to be clicked, as pointed by the red arrow in the figure above. This will lead to the window depicted in the figure below.

Figure C.3-4: "Related" tab in a Work Item search, with links to all related Specifications and Change Requests

The two links pointed by the red arrows in the figure above lead to the pages containing respectively all the Specifications and all the Change Requests (CRs) linked to this Feature.

Annex D:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-11 | TSG#94 |  |  |  |  | Initital draft, call for contributions | 0.1.0 |
| 2022-01 | - |  |  |  |  | Inclusion of the following summaries: RP-213003 WI Summary for Introduction of DL 1024QAM for NR FR1; rapporteur: Ericsson, Nokia Ericsson | 0.2.0 |

Potentially missing summaries:

| UID | Name | A c r o n y m | WG | WID | WI rapporteur name/company |
| --- | --- | --- | --- | --- | --- |
| 840050 | Enhancements for cyber-physical control applications in vertical domains | e C A V | S1 | SP-190310 | Bahr, Michael, Siemens AG |
| 910065 | Enhanced support of Non-Public Networks | e N P N | S2 | SP-200980 | Hedman, Peter, Ericsson |
| 840045 | Audio-Visual Service Production | A V P R O D | S1 | SP-190304 | Wagdin, Ian, BBC |
| 890035 | Enhancements to Application Architecture for the Mobile Communication System for Railways Phase 2 | e M O N A S T E R Y 2 | S6 | SP-191104 | Oettl, Martin, Nokia |
| 910052 | Enhancement to the 5GC LoCation Services-Phase 2 | 5 G _ e L C S _ p h 2 | S2 | SP-200082 | Ming Ai, CATT |
| 900060 | RAN aspects of NR positioning enhancements | N R _ p o s _ e n h | R1 | RP-210903 | Intel |
| 840047 | Communication Service Requirements for Critical Medical Applications | C M E D | S1 | SP-190306 | Lagrange, Mathieu, b<>com |
| 840083 | 5G Enhancement for UAVs | E A V | S1 | SP-190308 | Qun Wei, China Unicom |
| 920045 | Application layer support for Uncrewed Aerial System (UAS) | U A S A P P | S6 | SP-200988 | Niranth Amogh, Huawei Telecommunications India |
| 810049 | Remote Identification of Uncrewed Aerial Systems | I D _ U A S | S1 | SP-180771 | Eddy HALL, Qualcomm |
| 890034 | Integration of satellite components in the 5G architecture | 5 G S A T _ A R C H | S1 | SP-191335 | Cyril MICHEL, thales |
| 920069 | NB-IoT/eMTC support for Non-Terrestrial Networks | L T E _ N B I O T _ e M T C _ N T N | R1 | RP-211601 | MediaTek |
| 910059 | Support of Enhanced Industrial IIoT | I I o T | S2 | SP-200973 | Devaki Chandramouli, Nokia |
| 860044 | Additional enhancements for NB-IoT and LTE-MTC | N B _ I O T e n h 4 _ L T E _ e M T C 6 | R1 | RP-211340 | Huawei |
| 860051 | NR small data transmissions in INACTIVE state | N R _ S m a l l D a t a _ I N A C T I V E | R2 | RP-212594 | ZTE |
| 860045 | Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR | N R _ I I O T _ U R L L C _ e n h | R2 | RP-210854 | Nokia |
| 860042 | NR Sidelink enhancement | N R _ S L _ e n h | R1 | RP-202846 | LG Electronics |
| 910075 | Enhanced application layer support for V2X services | e V 2 X A P P | S6 | SP-200831 | Niranth Amogh, Huawei Telecommunications India |
| 910037 | Support of advanced V2X services - Phase 2 | e V 2 X A R C _ P h 2 | S2 | SP-210090 | LaeYoung Kim, LG Electronics |
| 920070 | UE RF requirements for Transparent Tx Diversity (TxD) for NR | N R _ R F _ T x D | R4 | RP-211940 | Qualcomm |
| 860040 | Further enhancements on MIMO for NR | N R _ f e M I M O | R1 | RP-212535 | Samsung |
| 860049 | Further Multi-RAT Dual-Connectivity enhancements | L T E _ N R _ D C _ e n h 2 | R2 | RP-201040 | Huawei |
| 860050 | Enhancements to Integrated Access and Backhaul (IAB) for NR | N R _ I A B _ e n h | R2 | RP-211548 | Qualcomm |
| 860052 | Enhanced eNB(s) architecture evolution for E-UTRAN and NG-RAN | L T E _ N R _ a r c h _ e v o _ e n h | R3 | RP-211409 | China Unicom |
| 890055 | Further enhancement on NR demodulation performance | N R _ d e m o d _ e n h 2 | R4 | RP-212636 | China Telecom |
| 890056 | Introduction of DL 1024QAM for NR frequency range 1 (FR1) | N R _ D L 1 0 2 4 Q A M _ F R 1 | R4 | RP-202886 | Ericsson |
| 890057 | NR RRM further enhancement | N R _ R R M _ e n h 2 | R4 | RP-202874 | Apple |
| 890058 | Enhanced NR support for high speed train scenario for frequency range 1 (FR1) | N R _ H S T _ F R 1 _ e n h | R4 | RP-210833 | CMCC |
| 890059 | Further enhancements of NR RF requirements for frequency range 2 (FR2) | N R _ R F _ F R 2 _ r e q _ e n h 2 | R4 | RP-212092 | Nokia |
| 890060 | NR support for high speed train scenario in frequency range 2 (FR2) | N R _ H S T _ F R 2 | R4 | RP-210800 | Samsung |
| 890061 | NR measurement gap enhancements | N R _ M G _ e n h | R4 | RP-211591 | MediaTek |
| 860053 | Enhancement of data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR and EN-DC | N R _ E N D C _ S O N _ M D T _ e n h | R3 | RP-193255 | CMCC |
| 910029 | Plug and connect support for management of Network Functions | P A C M A N | S5 | SP-210260 | Per Elmdahl, Ericsson |
| 870028 | Enhancements of Self-Organizing Networks (SON) for 5G networks | e S O N _ 5 G | S5 | SP-200194 | Joey Chou, Intel |
| 900038 | Multicast-broadcast services in 5G | 5 M B S | S2 | SP-201106 | Meng Li, Huawei |
| 850040 | Broadcast / Multicast requirements supporting Mission Critical Services in 5G | 5 M B S _ e M C | S1 | SP-190942 | Toobe, Jens, BDBOS |
| 860048 | NR multicast and broadcast services | N R _ M B S | R2 | RP-193248 | Huawei |
| 920051 | Mission Critical Services over 5GS | M C O v e r 5 G S | S6 | SP-200833 | Wendler, Ingo, UIC |
| 890039 | Mission Critical Data | e M C D a t a 3 | S6 | SP-191106 | Shih, Jerry, AT&T |
| 880020 | Protocol enhancements for Mission Critical Services | M C P r o t o c 1 7 | C1 | CP-202193 | AXELL, Jörgen |
| 820040 | Mission Critical system migration and interconnection | M C S M I _ C T | C1 | CP-190143 | Dom Lazara, Motorola Solutions |
| 890003 | CT aspects of Enhanced Mission Critical Communication Interworking with Land Mobile Radio Systems | e M C C I _ C T | C1 | eMCCI_CT | Mike Dolan, FirstNet |
| 880042 | Architecture for enabling Edge Applications | E D G E A P P | S6 | SP-200109 | Gupta, Nishant, Samsung |
| 910048 | Enhancement of support for Edge Computing in 5G Core network | e E D G E _ 5 G C | S2 | SP-201107 | Hui Ni, Huawei Technologies |
| 920019 | Edge Computing Management | E C M | S5 | SP-210388 | Deepanshu Gautam, Samsung |
| 930034 | Charging aspects of Edge Computing | E D G E _ C H | S5 | SP-210861 | Yao, Yizhi, Intel |
| 920058 | Rel-17 Enhancements of 3GPP Northbound Interfaces and Application Layer APIs | N B I 1 7 | C1 | CP-211197 | Yali Yan, Huawei |
| 920012 | Enhancement of 5G PCC related services in Rel-17 | e n 5 G P c c S e r 1 7 | C3 | CP-211193 | Xiaoyun Zhou, Huawei |
| 900032 | Enhancement of Network Slicing Phase 2 | e N S _ P h 2 | S2 | SP-200976 | ZTE, Jinguo Zhu |
| 911007 | Enhancement of RAN slicing for NR | N R _ s l i c e | R2 | RP-212534 | CMCC |
| 840049 | Support for Multi-USIM Devices | M U S I M | S1 | SP-190309 | Liao, Ellen C, Intel |
| 860063 | Support for Multi-SIM devices for LTE/NR | L T E _ N R _ M U S I M | R2 | RP-212610 | Vivo |
| 930030 | Support of the 5GMSG Service | F S _ 5 G M A R C H | S6 | SP-200835 | Liu, Yue, China Mobile |
| 930040 | Application Architecture for MSGin5G Service | 5 G M A R C H | S6 | SP-200832 | Liu, Yue, China Mobile |
| 930002 | IMS voice service support and network usability guarantee for UE s E-UTRA capability disabled scenario in SA 5GS | I N G _ 5 G S | ct | CP-212231 | Mingxue Li, China Telecom |
| 930031 | User Consent for 3GPP services | F S _ U C 3 S | S3 | SP-200885 | Rong Wu, Huawei Technologies |
| 930032 | Improved support for NSA in the service-based management architecture | N S A _ S B M A | S5 | SP-211121 | Lan Zou, Huawei |
| 930033 | Access control for management service | M S A C | S5 | SP-210859 | Jing Ping, Nokia |
| 920060 | NPN support of PWS | N P N _ P W S | S1 | SP-210585 | Francesco Pica, Qualcomm |
| 910007 | Start of Pause of Charging via User Plane | S P O C U P | C4 | CP-210076 | YONG, YANG, Ericsson, frank |
| 910030 | File Management | F I M A | S5 | SP-210135 | Pollakowski, Olaf, Nokia |
| 910024 | Enhancements of 3GPP profiles for cryptographic algorithms and security protocols | e C r y p t P r | S3 | SP-210107 | Pinar Comak, Ericsson |
| 920027 | AKMA TLS protocol profiles | A K M A _ T L S | S3 | SP-210424 | Escott, Adrian, Qualcomm |
| 910025 | User Plane Integrity Protection for LTE | U P I P _ S E C _ L T E | S3 | SP-210105 | Evans, Tim, Vodafone |
| 900002 | Restoration of PDN Connections in PGW-C/SMF Set | R P C P S E T | C4 | CP-203020 | LANDAIS, Bruno, Nokia |
| 900037 | Enhancement of Inter-PLMN Roaming | E o I P R | C4 | CP-203021 | Caixia Qi, Huawei |
| 911037 | Reliable Data Service Serialization Indication | R D S S I | S2 | SP-190446 | Starsinic, Michael, Convida Wireless LLC |
| 900021 | Management Aspects of 5G Network Sharing | M A N S | S5 | SP-201080 | Mingrui Sun, China Unicom |
| 900019 | Adapting BEST for use in 5G networks | B E S T _ 5 G | S3 | SP-201020 | Keesmaat, Iko, KPN |
| 920049 | Enhanced Service Enabler Architecture Layer for Verticals | e S E A L | S6 | SP-200987 | Basavaraj (Basu) Pattan, Samsung |
| 900033 | Access Traffic Steering, Switch and Splitting support in the 5G system architecture; Phase 2 | A T S S S _ P h 2 | S2 | SP-200977 | Apostolis Salkintzis, Lenovo |
| 900031 | Enablers for Network Automation for 5G - phase 2 | e N A _ P h 2 | S2 | SP-200975 | TBD |
| 900030 | Proximity based Services in 5GS | 5 G _ P r o S e | S2 | SP-200972 | TBD |
| 911005 | NR Sidelink Relay | N R _ S L _ r e l a y | R2 | RP-212601 | OPPO |
| 890030 | Authentication and key management for applications based on 3GPP credential in 5G | A K M A | S3 | SP-190711 | Xiaoting Huang, China Mobile |
| 890049 | Enhancement of Private Network support for NG-RAN | N G _ R A N _ P R N _ e n h | R3 | RP-212585 | China Telecom |
| 840046 | Multimedia Priority Service (MPS) Phase 2 | M P S 2 | S1 | SP-190305 | Singh, Ray P; Perspecta Labs |
| 890011 | Mission critical security enhancements phase 2 | M C X S e c 2 | S3 | SP-200879 | Woodward, Tim, Motorola Solutions, Inc, |
| 890070 | PAP/CHAP protocols usage in 5GS | P A P _ C H A P | ct | CP-202251 | Mingxue Li, China Telecom |
| 890001 | Service-based support for SMS in 5GC | S M S _ S B I | C1 | CP-212023 | HAOUARI, Wafa, Orange |
| 840048 | Enhanced Relays for Energy eFficiency and Extensive Coverage | R E F E C | S1 | SP-190307 | Norp, Toon, KPN |
| 840044 | Network Controlled Interactive Service | N C I S | S1 | SP-190303 | Ning YANG, oppo |
| 840019 | 5G System Enhancement for Advanced Interactive Services | 5 G _ A I S | S2 | SP-190564 | Lei Yixue, Tencent |
| 890009 | Points for 8K VR 360 Video over 5G | 8 K _ V R _ 5 G | S4 | SP-200667 | Ozgur Oyman, Company: Intel |
| 880043 | Multi-Device and multi-identity Enhancements (MuDE) | M u D E | S1 | SP-190823 | Adrian.buckley@vivo.com |
| 920055 | Best Practice of Packet Forwarding Control Protocol (PFCP) | B E P o P | ct | CP-212024 | Song Yue, China Mobile |
| 880029 | Enhancement of Handover Optimization | E _ H O O | S5 | SP-200466 | Elmdahl, Per, Ericsson |
| 880028 | Management data collection control and discovery | M A D C O L | S5 | SP-200465 | Pollakowski, Olaf, Nokia |
| 880026 | Management of the enhanced tenant concept | e M E M T A N E | S5 | SP-200463 | Zhu, Lei, Huawei |
| 880025 | Enhancements of 5G performance measurements and KPIs | e P M _ K P I _ 5 G | S5 | SP-200462 | Yizhi Yao, Intel |
| 880012 | Extension for headset interface tests of UE | H I n T | S4 | SP-200398 | Jan Reimes, HEAD acoustics GmbH, |
| 840038 | MC services support on IOPS mode of operation | M C I O P S | S6 | SP-190944 | Camilo.solano@ericsson.com |
| 840016 | Supporting Flexible Local Area Data Network | F L A D N | S2 | SP-190563 | LG Electronics, Hyunsook Kim |
| 820003 | Support of Immersive Teleconferencing and Telepresence for Remote Terminals | I T T 4 R T | S4 | SP-180985 | Ozgur Oyman, Intel |
| 830005 | Terminal Audio quality performance and Test methods for Immersive Audio Services | A T I A S | S4 | SP-190040 | Stéphane Ragot, Orange |
| 860012 | Handsets Featuring Non-Traditional Earpieces | H a N T E | S4 | SP-190989 | Schevciw, Andre, Qualcomm |
| 920008 | 5GMS AF Event Exposure | E V E X | S4 | SP-210374 | Lo, Charles, Qualcomm Inc., |
| 920009 | Edge Extensions to the 5G Media Streaming Architecture | 5 G M S _ E D G E | S4 | SP-210375 | Bouazizi, Imed, Qualcomm Inc. |
| 920010 | 5G Multicast-Broadcast User Service Architecture and related 5GMS Extensions | 5 M B U S A | S4 | SP-210376 | TAN, PENG, TELUS |
| 920011 | 8K Television over 5G | 8 K _ T V _ 5 G | S4 | SP-210381 | Thomas Stockhammer |
| 830017 | Verification-modified Calling Name Display | V M O D _ D I S P L A Y | S1 | SP-190085 | Hala Mowafy, Ericsson |
| 850038 | Enhanced Calling Name Service Analytics Interworking | e C N A M _ A n | S1 | SP-190940 | Hala.Mowafy@ericsson.com |
| 880049 | Enhancement for the 5G Control Plane Steering of Roaming for UE in Connected mode | e C P S O R _ C O N | S1 | CP-210148 | Minokuchi, Atsushi, NTT DOCOMO |
| 910090 | Integration of Generic Bootstrapping Architecture (GBA) into 5GC | G B A _ 5 G | S3 | SP-190714 | Vlasios Tsiatsis, Ericsson |
| 770024 | EVS Codec Extension for Immersive Voice and Audio Services | I V A S _ C o d e c | S4 | SP-170611 | Bin Wang, Huawei Technologies Co Ltd |
| 850045 | Support for Minimization of service Interruption | M I N T | S1 | SP-190814 | SungDuck Chun; LG Electronics |
| 860008 | IMS emergency support for Stand-alone Non-Public Network (SNPN) | I E S N P N | S1 | SP-191038 | Wong, Curt; |
| 870023 | Management of non-public networks (NPN) | O A M _ N P N | S5 | SP-200189 | ZHANG, Kai, Huawei |
| 860016 | Assurance Specification for IMS | S C A S _ I M S | S3 | SP-191128 | Bo Zhang, Huawei Technologies |
| 870020 | Security Assurance Specification for 5G (eSCAS_5G) | e S C A S _ 5 G | S3 | SP-200149 | Rong Wu, Huawei Technologies Co. |
| 870022 | Enhancements on EE for 5G networks | E E 5 G P L U S | S5 | SP-200188 | Jean-Michel CORNILY, ORANGE |
| 870024 | Enhancement on Management Aspects of 5G Service-Level Agreement | E M A 5 S L A | S5 | SP-210860 | SHI, Xiaonan, CMCC |
| 870025 | Management of MDT enhancement in 5G | e _ 5 G M D T | S5 | SP-200191 | Ayani, Zhulia, Ericsson |
| 870026 | Additional NRM features | a d N R M | S5 | SP-200192 | Jing Ping, Nokia |
| 870027 | Enhancement of QoE Measurement Collection | e Q o E | S5 | SP-200193 | Petersen, Robert, Ericsson |
| 870030 | Enhanced Closed loop SLS Assurance | e C O S L A | S5 | SP-200196 | Groenendijk, Jan, Ericsson |
| 840028 | IMS Charging in 5G System Architecture | 5 G S I M S C H | S5 | SP-190367 | Jahangir, Zeeshan, T-Mobile USA |
| 810027 | Intent driven management service for mobile network | I D M S _ M N | S5 | SP-180899 | Zou Lan, Huawei |
| 890012 | Enhancements to User Plane Integrity Protection Support in 5GS | e U P I P _ S E C | S3 | SP-200719 | Anand Palanigounder, Qualcomm |
| 820035 | Discovery of management services in 5G | 5 G D M S | S5 | SP-200770 | Brendan Hassett, Huawei Technologies Sweden AB |
| 860024 | Network policy managementfor 5G mobile networks | N P M | S5 | SP-191211 | China Mobile Shasha Guo |
| 860025 | Lawful Interception Rel-17 | L I 1 7 | S3 | SP-190983 | Alex Leadbeater, BT |
| 870003 | IMS Optimization for HSS Group ID in an SBA environment | T E I 1 7 _ I M S G I D | S2 | SP-200452 | Foti, George, Ericsson |
| 910044 | Same PCF Selection For AMF and SMF | T E I 1 7 _ S P S F A S | S2 | SP-200447 | Heng, Nie, China Telecom |
| 930036 | System enhancement for Redundant PDU Session | T E I 1 7 _ S E _ R P S | S2 | SP-200448 | Devaki Chandramouli |
| 910078 | Support for Signed Attestation for Priority and Emergency Sessions | T E I 1 7 _ S A P E S | S2 | SP-200453 | Foti, George, Ericsson |
| 910081 | Dynamically Changing AM Policies in the 5GC | T E I 1 7 _ D C A M P | S2 | SP-200446 | Pancorbo, Belen (Ericsson) |
| 870006 | IP address pool information from UDM | T E I 1 7 _ I P U | S2 | SP-200454 | Stefan Rommer |
| 910084 | Dynamic management of group-based event monitoring | T E I 1 7 _ G E M | S2 | SP-200455 | Qian XB Chen, Ericsson |
| 911034 | IMS Stage-3 IETF Protocol Alignment | I M S P r o t o c 1 7 | ct | CP-201167 | Gkatzikis, Lazaros(Nokia) |
| 880032 | Packet Flow Description (PFD) management enhancement | p f d M a n E n h | C3 | CP-210183 | Xiaoyun Zhou, Huawei |
| 880013 | Service Based Interface Protocol Improvements Release 17 | S B I P r o t o c 1 7 | C3 | CP-211088 | Song Yue, China Mobile |
| 911008 | NR QoE magement and optimizations for diverse services | N R _ Q o E | R3 | RP-211406 | China Unicom |
| 911009 | Rel-17 Power Class 2 UE for NR inter-band CA with or without SUL configurations with x (16>=x>2) bands DL and y (y=1, 2) bands UL | N R _ U E _ P C 2 _ R 1 7 _ C A D C _ S U L _ x B D L _ y B U L | R4 | RP-212182 | Huawei |
| 911010 | Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC) | N R _ F R 1 _ T R P _ T R S | R4 | RP-212030 | Vivo |
| 911012 | High power UE (power class 1.5) for NR band n79 | N R _ U E _ P C 1 _ 5 _ n 7 9 | R4 | RP-210843 | CMCC |
| 911015 | Introduction of FR2 FWA (Fixed Wireless Access) UE with maximum TRP of 23dBm for band n259 | N R _ F R 2 _ F W A _ B n 2 5 9 | R4 | RP-210875 | SoftBank Corp., KDDI, NTT DOCOMO, Rakuten Mobile, Qualcomm |
| 911006 | NR Uplink Data Compression (UDC) | N R _ U D C | R2 | RP-211203 | CATT |
| 880078 | Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) requirements for NR UEs | N R _ M I M O _ O T A | R4 | RP-201800 | CAICT |
| 870064 | Introduction of FR2 FWA (Fixed Wireless Access) UE with maximum TRP (Total Radiated Power) of 23dBm for band n257 and n258 | N R _ F R 2 _ F W A _ B n 2 5 7 _ B n 2 5 8 | R4 | RP-202565 | Softbank |
| 880092 | Adding channel bandwidth support to existing NR bands | N R _ b a n d s _ R 1 7 _ B W s | R4 | RP-212531 | Ericsson |
| 880093 | Introduction of channel bandwidths 35MHz and 45MHz for NR FR1 | N R _ F R 1 _ 3 5 M H z _ 4 5 M H z _ B W | R4 | RP-211386 | Huawei |
| 880097 | SAR schemes for UE power class 2 (PC2) for NR inter-band Carrier Aggregation and supplemental uplink (SUL) configurations with 2 bands UL | N R _ S A R _ P C 2 _ i n t e r B _ S U L _ 2 B U L | R4 | RP-212530 | China Telecom |
| 900061 | NR coverage enhancements | N R _ c o v _ e n h | R1 | RP-211566 | China Telecom |
| 900062 | Support of reduced capability NR devices | N R _ r e d c a p | R1 | RP-211574 | Ericsson |
| 900167 | Introduction of bandwidth combination set 4 (BCS4) for NR | N R _ B C S 4 - C o r e | R4 | RP-202832 | Ericsson |
| 930056 | Increasing UE power high limit for CA and DC | P o w e r _ L i m i t _ C A _ D C | R4 | RP-212622 | Qualcomm |
| 930057 | Introduction of upper 700MHz A block E-UTRA band for the US | L T E _ u p p e r _ 7 0 0 M H z _ A | R4 | RP-212618 | Puloli |
| 930058 | High power UE (power class 2) for one NR FDD band | N R _ P C 2 _ U E _ F D D | R4 | RP-212633 | China Unicom |
| 920072 | LTE/NR spectrum sharing in Band 34/n34 and Band 39/n39 | D S S _ L T E _ B 3 4 _ N R _ B n 3 4 _ L T E _ B 3 9 _ N R _ B n 3 9 | R4 |  | CMCC |
| 920073 | High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands | L T E _ N R _ H P U E _ F W V M | R4 | RP-212533 | Nokia |
| 920074 | Rel-17 Dual Connectivity (DC) of x bands (x=1,2) LTE inter-band CA (xDL1UL) and 4 bands NR inter-band CA (4DL1UL) | D C _ R 1 7 _ x B L T E _ 4 B N R _ y D L 2 U L | R4 | RP-212606 | Huawei |
| 860041 | Extending current NR operation to 71GHz | N R _ e x t _ t o _ 7 1 G H z | R1 | RP-212637 | Qualcomm |
| 870063 | Introduction of NR band n13 | N R _ n 1 3 | R4 | RP-200480 | Huawei |
| 880082 | Introduction of NR 47GHz band | N R _ 4 7 G H z _ b a n d | R4 | RP-212528 | Ericsson |
| 880084 | Introduction of NR band 24 | N R _ b a n d _ n 2 4 | R4 | RP-211505 | Ligado Networks |
| 890050 | Introduction of 6GHz NR licensed bands | N R _ 6 G H z | R4 | RP-202844 | Huawei |
| 920075 | Introduction of operation in full unlicensed band 5925-7125MHz for NR | N R _ 6 G H z _ u n l i c _ f u l l | R4 | RP-212302 | Apple |
| 890051 | Introduction of lower 6GHz NR unlicensed operation for Europe | N R _ 6 G H z _ u n l i c _ E U | R4 | RP-212625 | Nokia |
| 900065 | Introduction of NR band n67 | N R _ n 6 7 | R4 | RP-202829 | Ericsson |
| 900066 | Introduction of NR band n85 | N R _ n 8 5 | R4 | RP-210707 | Ericsson |
| 911020 | New bands and bandwidth allocation for 5G terrestrial broadcast - part 1 | L T E _ t e r r _ b c a s t _ b a n d s _ p a r t 1 | R1 | RP-211144 | EBU |