3GPP TS 23.288 V16.2.0 (2019-12)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Architecture enhancements for 5G System (5GS) to support network data analytics services

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

3GPP, 5G, Architecture, Network, Automation

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	6

2	References	6

3	Definitions and abbreviations	7

3.1	Definitions	7

3.2	Abbreviations	7

4	Reference Architecture for Data Analytics	7

4.1	General	7

4.2	Non-roaming architecture	7

4.3	Roaming architecture	8

5	Network Data Analytics Functional Description	8

5.1	General	8

5.2	NWDAF Discovery and Selection	8

6	Procedures to Support Network Data Analytics	9

6.0	General	9

6.1	Procedures for analytics exposure	9

6.1.1	Analytics Subscribe/Unsubscribe	9

6.1.1.1	Analytics subscribe/unsubscribe by NWDAF service consumer	9

6.1.1.2	Analytics subscribe/unsubscribe by AFs via NEF	9

6.1.2	Analytics Request	11

6.1.2.1	Analytics request by NWDAF service consumer	11

6.1.2.2	Analytics request by AFs via NEF	11

6.1.3	Contents of Analytics Exposure	12

6.2	Procedures for Data Collection	13

6.2.1	General	13

6.2.2	Data Collection from NFs	14

6.2.2.1	General	14

6.2.2.2	Procedure for Data Collection from NFs	16

6.2.2.3	Procedure for Data Collection from AF via NEF	16

6.2.2.4	Procedure for Data Collection from NRF	18

6.2.2.5	Usage of Exposure framework by the NWDAF for Data Collection	18

6.2.3	Data Collection from OAM	19

6.2.3.1	General	19

6.2.3.2	Procedure for data collection from OAM	19

6.2.4	Correlation between network data and service data	20

6.3	Slice load level related network data analytics	20

6.3.1	General	20

6.3.2	Void	20

6.3.2A	Input data	21

6.3.3	Void	21

6.3.3A	Output analytics	21

6.4	Observed Service experience related network data analytics	21

6.4.1	General	21

6.4.2	Input Data	22

6.4.3	Output Analytics	23

6.4.4	Procedures to request Service Experience for an Application	24

6.4.5	Procedures to request Service Experience for a Network Slice	26

6.5	NF load analytics	26

6.5.1	General	26

6.5.2	Input data	27

6.5.3	Output analytics	28

6.5.4	Procedures	28

6.6	Network Performance Analytics	30

6.6.1	General	30

6.6.2	Input Data	30

6.6.3	Output Analytics	31

6.6.4	Procedures	32

6.7	UE related analytics	33

6.7.1	General	33

6.7.2	UE mobility analytics	33

6.7.2.1	General	33

6.7.2.2	Input Data	33

6.7.2.3	Output Analytics	34

6.7.2.4	Procedures	35

6.7.3	UE Communication Analytics	37

6.7.3.1	General	37

6.7.3.2	Input Data	37

6.7.3.3	Output Analytics	37

6.7.3.4	Procedures	38

6.7.4	Expected UE behavioural parameters related network data analytics	40

6.7.4.1	General	40

6.7.4.2	Input Data	40

6.7.4.3	Output Analytics	40

6.7.4.4	Procedures	41

6.7.4.4.1	NWDAF-assisted expected UE behavioural analytics	41

6.7.5	Abnormal behaviour related network data analytics	41

6.7.5.1	General	41

6.7.5.2	Input Data	42

6.7.5.3	Output Analytics	43

6.7.5.4	Procedure	45

6.8	User Data Congestion Analytics	46

6.8.1	General	46

6.8.2	Input data	47

6.8.3	Output analytics	47

6.8.4	Procedures	48

6.8.4.1	Procedure for one-time or continuous reporting of analytics for user data congestion in a geographic area	48

6.8.4.2	Procedure for one-time or continuous reporting of analytics for user data congestion for a specific UE	49

6.9	QoS Sustainability Analytics	51

6.9.1	General	51

6.9.2	Input data	52

6.9.3	Output analytics	52

6.9.4	Procedures	53

7	Nnwdaf Services Description	54

7.1	General	54

7.2	Nnwdaf_AnalyticsSubscription Service	55

7.2.1	General	55

7.2.2	Nnwdaf_AnalyticsSubscription_Subscribe service operation	55

7.2.3	Nnwdaf_AnalyticsSubscription_Unsubscribe service operation	56

7.2.4	Nnwdaf_AnalyticsSubscription_Notify service operation	56

7.3	Nnwdaf_AnalyticsInfo service	56

7.3.1	General	56

7.3.2	Nnwdaf_AnalyticsInfo_Request service operation	56

Annex A (informative):	Change history	57

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document defines the Stage 2 architecture enhancements for 5G System (5GS) to support network data analytics services in 5G Core network.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[5]	Void.

[6]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[7]	3GPP TS 28.550: "Management and orchestration; Performance Assurance".

[8]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[9]	3GPP TS 28.545: "Management and orchestration; Fault Supervision (FS)".

[10]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[11]	ITU-T Recommendation P.1203.3: "Parametric bitstream-based quality assessment of progressive download and adaptive audiovisual streaming services over reliable transport - Quality integration module".

[12]	3GPP TS 38.215: "NR; Physical layer measurements".

[13]	Void.

[14]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[15]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[16]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[17]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane Nodes".

[18]	3GPP TS 29.510: "5G System; Network function repository services; Stage 3".

[19]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], TS 23.501 [2], TS 23.502 [3] and TS 23.503 [4] apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

MDT	Minimization of Driving Tests

# 4	Reference Architecture for Data Analytics

## 4.1	General

The NWDAF is part of the architecture specified in TS 23.501 [2] and uses the mechanisms and interfaces specified for 5GC in TS 23.501 [2] and OAM services (see clause 6.2.3.1).

The NWDAF interacts with different entities for different purposes:

-	Data collection based on subscription to events provided by AMF, SMF, PCF, UDM, AF (directly or via NEF), and OAM;

-	Retrieval of information from data repositories (e.g. UDR via UDM for subscriber-related information);

-	Retrieval of information about NFs (e.g. from NRF for NF-related information);

-	On demand provision of analytics to consumers, as specified in clause 6.

A single instance or multiple instances of NWDAF may be deployed in a PLMN. In case multiple NWDAF instances are deployed, the architecture supports deploying the NWDAF as a central NF, as a collection of distributed NFs, or as a combination of both.

NOTE 1:	When multiple NWDAFs exist, not all of them need to be able to provide the same type of analytics results, i.e., some of them can be specialized in providing certain types of analytics. An Analytics ID information element is used to identify the type of supported analytics that NWDAF can generate.

NOTE 2:	NWDAF instance(s) can be collocated with a 5GS NF.

## 4.2	Non-roaming architecture

As depicted in Figure 4.2-1, the 5G System architecture allows NWDAF to collect data from any NF. The NWDAF belongs to the same PLMN where the network function that notifies the data is located.

Figure 4.2-1: Data Collection architecture from any NF

The Nnf interface is defined for the NWDAF to request subscription to data delivery for a particular context, to cancel subscription to data delivery and to request a specific report of data for a particular context.

The 5G System architecture allows NWDAF to retrieve the management data from OAM by invoking the existing OAM services.

As depicted in Figure 4.2-2, the 5G System architecture allows any NF to request network analytics information from NWDAF. The NWDAF belongs to the same PLMN where the network function that consumes the analytics information is located.

Figure 4.2-2: Network Data Analytics Exposure architecture

The Nnwdaf interface is defined for the network functions, to request subscription to network analytics delivery for a particular context, to cancel subscription to network analytics delivery and to request a specific report of network analytics for a particular context.

## 4.3	Roaming architecture

The interactions between the NWDAF and the other NFs are only considered in the same PLMN case.

Roaming architecture does not apply in this release of the specification.

# 5	Network Data Analytics Functional Description

## 5.1	General

The NWDAF (Network Data Analytics Function) provides analytics to 5GC NFs, and OAM as defined in clause 7.

Analytics information are either statistical information of the past events, or predictive information.

Different NWDAF instances may be present in the 5GC, with possible specializations per type of analytics. The capabilities of a NWDAF instance are described in the NWDAF profile stored in the NRF.

In order to support NFs that are consumers of analytics with the discovery of a NWDAF instance that is able to provide some specific type of analytics, each NWDAF instance should provide the list of Analytics ID(s) that it supports when registering to the NRF, in addition to other NRF registration elements of the NF profile. Other NFs requiring the discovery of an NWDAF instance that provides support for some specific type of analytics may query the NRF and include the Analytics ID(s) that identifies the desired type of analytics for that purpose.

The consumers i.e. 5GC NFs and OAM decide how to use the data analytics provided by NWDAF.

The interactions between 5GC NF(s) and the NWDAF take place within a PLMN.

The NWDAF has no knowledge about NF application logic. The NWDAF may use subscription data but only for statistical purpose.

## 5.2	NWDAF Discovery and Selection

The NWDAF service consumer selects an NWDAF that supports requested analytics information by using the NWDAF discovery principles defined in clause 6.3.13, TS 23.501 [2].

# 6	Procedures to Support Network Data Analytics

## 6.0	General

This clause specifies procedures to support network data analytics function.

Clause 6.1 and clause 6.2 specify generic procedures which apply to all type of analytics, while clause 6.3 and onwards specify procedures specific to some type of analytics.

## 6.1	Procedures for analytics exposure

### 6.1.1	Analytics Subscribe/Unsubscribe

#### 6.1.1.1	Analytics subscribe/unsubscribe by NWDAF service consumer

This procedure is used by any NWDAF service consumer (e.g. including NFs/OAM) to subscribe/unsubscribe at NWDAF to be notified on analytics information, using Nnwdaf_AnalyticsSubscription service defined in clause 7.2. This service is also used by an NWDAF service consumer to modify existing analytics subscription(s). Any entity can consume this service as defined in clause 7.2.

Figure 6.1.1.1-1: Network data analytics Subscribe/unsubscribe

1.	The NWDAF service consumer subscribes to or cancels subscription to analytics information by invoking the Nnwdaf_AnalyticsSubscription_Subscribe/ Nnwdaf_AnalyticsSubscription_Unsubscribe service operation. The parameters that can be provided by the NWDAF service consumer are listed in clause 6.1.3.

When a subscription to analytics information is received, the NWDAF determines whether triggering new data collection is needed.

If the service invocation is for a subscription modification, the NF service consumer includes an identifier (Subscription Correlation ID) to be modified in the invocation of Nnwdaf_AnalyticsSubscription_Subscribe.

2.	If NWDAF service consumer subscribes to analytics information, the NWDAF notifies the NWDAF service consumer with the analytics information by invoking Nnwdaf_AnalyticsSubscription_Notify service operation, based on the request from the NWDAF service consumer, e.g. Analytics Reporting Parameters.

#### 6.1.1.2	Analytics subscribe/unsubscribe by AFs via NEF

The analytics exposure to AFs may be performed via NEF by using analytics subscription to NWDAF.

Figure 6.1.1.2-1 illustrates the interaction between AF and NWDAF performed via the NEF.

Figure 6.1.1.2-1: Procedure for analytics subscribe/unsubscribe by AFs via NEF

0.	NEF controls the analytics exposure mapping among the AF identifier with allowed Analytics ID, and associated inbound restrictions (i.e, applied to subscription of the Analytics ID for an AF) and/or outbound restrictions (i.e., applied to notification of Analytics ID to an AF).

In this Release, AF is configured with the appropriated NEF to subscribe to analytics information, the allowed Analytics ID(s), and with allowed inbound restrictions (i.e., parameters and/or parameter values) for subscription to each Analytics ID.

1.	The AF subscribes to or cancels subscription to analytics information via NEF by invoking the Nnef_AnalyticsExposure_Subscribe/ Nnef_AnalyticsExposure_Unsubscribe service operation defined in TS 23.502 [3]. If the AF wants to modify an existing analytics subscription at NEF, it includes an identifier (Subscription Correlation ID) to be modified in the invocation of Nnef_AnalyticsExposure_Subscribe. If the analytics information subscription is authorized by the NEF, the NEF proceeds with the steps below.

2.	Based on the request from the AF, the NEF subscribes to or cancels subscription to analytics information by invoking the Nnwdaf_AnalyticsSubscription_Subscribe/ Nnwdaf_AnalyticsSubscription_Unsubscribe service operation.

If the parameters and/or parameters values of the AF request comply with the inbound restriction in the analytics exposure mapping, NEF forwards in the subscription to NWDAF service the Analytics ID, parameters and/or parameters values from the AF request.

If the request from AF does not comply with the restrictions in the analytics exposure mapping, NEF may apply restrictions to the subscription request to NWDAF (e.g., restrictions to parameters or parameter values of the Nnwdaf_AnalyticsSubscription_Subscribe service operations), based on operator configuration and/or may apply parameter mapping (e.g. geo coordinate mapping to TA(s)/Cell-id(s)).

The NEF records the association of the analytics request from the AF and the analytics request sent to the NWDAF.

The NEF selects an NWDAF that supports analytics information requested by the AF using the NWDAF discovery procedure defined in TS 23.501 [2].

If the AF request is for a modification of the existing analytics subscription(s), the NEF invokes Nnwdaf_AnalyticsSubscription_Subscribe to modify the analytics subscription identified by an identifier (Subscription Correlation ID) associated with the AF.

3.	If the NEF has subscribed to analytics information, the NWDAF notifies the NEF with the analytics information by invoking Nnwdaf_AnalyticsSubscription_Notify service operation.

4.	If the NEF receives the notification from the NWDAF, the NEF notifies the AF with the analytics information by invoking Nnef_AnalyticsExposure_Notify service operation defined in TS 23.502 [3]. NEF may apply outbound restrictions to the notifications to AFs (e.g., restrictions to parameters or parameter values of the Nnef_AnalyticsExposure_Notify service operation) based on analytics exposure mapping and may apply parameter mapping for external usage (e.g. TA(s), Cell-id(s) to geo coordinate).

### 6.1.2	Analytics Request

#### 6.1.2.1	Analytics request by NWDAF service consumer

This procedure is used by the NWDAF service consumer (e.g. including NFs/OAM) to request and get from NWDAF analytics information, using Nnwdaf_AnalyticsInfo service defined in clause 7.3.

Figure 6.1.2.1-1: Network data analytics Request

1.	The NWDAF service consumer requests analytics information by invoking Nnwdaf_AnalyticsInfo_Request service operation. The parameters that can be provided by the NWDAF service consumer are listed in clause 6.1.3.

When a request for analytics information is received, the NWDAF determines whether triggering new data collection is needed.

2.	The NWDAF responds with analytics information to the NWDAF service consumer.

#### 6.1.2.2	Analytics request by AFs via NEF

The analytics exposure to AFs may be performed via NEF by using analytics request to NWDAF.

Figure 6.1.2.2-1 illustrates the interaction between AF and NWDAF performed via the NEF.

Figure 6.1.2.2-1: Procedure for analytics request by AFs via NEF

0.	NEF controls the analytics exposure mapping among the AF identifier with allowed Analytics ID(s), and associated inbound restrictions (i.e., applied to the Analytics ID requested by AF, and/or outbound restrictions (i.e., applied to the response of Analytics ID to AF).

In this Release, AF is configured, e.g., via static OAM configuration, with the appropriated NEF to subscribe to analytics information, the allowed Analytics ID(s), and with allowed inbound restrictions (i.e., parameters and/or parameter values) for requesting each Analytics ID.

1.	The AF requests to receive analytics information via NEF by invoking the Nnef_AnalyticsExposure_Fetch service operation defined in TS 23.502 [3]. If the analytics information request is authorized by the NEF, the NEF proceeds with the steps below.

2.	Based on the request from the AF, the NEF requests analytics information by invoking the Nnwdaf_AnalyticsInfo_Request service operation.

If the parameters and/or parameters values of the AF request comply with the restriction in the analytics exposure mapping, NEF forwards in the subscription to NWDAF service the Analytics ID, parameters and/or parameters values from AF in the request to NWDAF.

If the request from AF does not comply with the restrictions in the analytics exposure mapping, NEF may apply restrictions to the request to NWDAF (e.g., restrictions to parameters or parameter values of the Nnwdaf_AnalyticsInfo_Request service operations) based on operator configuration and/or may apply parameter mapping (e.g. geo coordinate mapping to TA(s), Cell-id(s)).

The NEF records the association of the analytics request from the AF and the analytics request sent to the NWDAF.

The NEF selects an NWDAF that supports analytics information requested by the AF using the NWDAF discovery procedure defined in TS 23.501 [2].

3.	The NWDAF responds with the analytics information to the NEF.

4.	The NEF responds with the analytics information to the AF. NEF may apply restrictions to the response to AFs (e.g., restrictions to parameters or parameter values of the Nnef_AnalyticsExposure_Fetch response service operation) based on operator configuration.

### 6.1.3	Contents of Analytics Exposure

The consumers of the Nnwdaf_AnalyticsSubscription or Nnwdaf_AnalyticsInfo service operations described in clause 7 may provide the following input parameters listed below.

-	A list of Analytics ID(s): identifies the requested analytics.

-	Analytics Filter Information: indicates the conditions to be fulfilled for reporting Analytics Information. This set of optional parameter types and values enables to select which type of analytics information is requested. Analytics Filter Information are defined in procedures.

-	Target of Analytics Reporting: indicates the object(s) for which Analytics information is requested, entities such as specific UEs, a group of UE(s) or any UE (i.e. all UEs).

-	(Only for Nnwdaf_AnalyticsSubscription) A Notification Target Address (+ Notification Correlation ID) as defined in TS 23.502 [3] clause 4.15.1, allowing to correlate notifications received from NWDAF with this subscription.

-	Analytics Reporting Information with the following parameters:

-	(Only for Nnwdaf_AnalyticsSubscription) Analytics Reporting Parameters as per Event Reporting parameters defined in Table 4.15.1-1, TS 23.502 [3];

-	(Only for Nnwdaf_AnalyticsSubscription) Reporting Thresholds, which indicate conditions on the level of each requested analytics that when reached shall be notified by the NWDAF;

-	Analytics target period: time interval [start..end], either in the past (both start time and end time in the past) or in the future (both start time and end time in the future). An Analytics target period in the past is a request or subscription for statistics. An Analytics target period in the future is a request or subscription for predictions. The time interval is expressed with actual start time and actual end time (e.g. via UTC time). When the Analytics Reporting Parameters indicate a periodic reporting mode, the time interval can also be expressed as positive or negative offsets to the reporting time. By setting start time and end time to the same value, the consumer of the analytics can request analytics or subscribe to analytics for a specific time rather than for a time interval.

-	Preferred level of accuracy of the analytics (e.g. Low/High).

-	(Only for Nnwdaf_AnalyticsInfo_Request) Time when analytics information is needed (if applicable). If the time is reached the consumer does not need to wait for the analytics information any longer, yet the NWDAF may send an error response to the consumer.

NOTE:	The feasibility of the parameter "Time when analytics are needed" will be checked by stage 3.

The NWDAF provides to the consumer of the Nnwdaf_AnalyticsSubscription or Nnwdaf_AnalyticsInfo service operations described in clause 7, the output information listed below:

-	(Only for Nnwdaf_AnalyticsSubscription) The Notification Correlation Information.

-	For each Analytics ID the analytics information in the requested Analytics target period.

-	In addition, the following additional information:

-	Timestamp of analytics generation, which allows consumers to decide until when the received information shall be used. For instance, an NF can deem a received notification from NWDAF for a given feedback as invalid based on this timestamp;

-	Validity period, which defines the time period for which the analytics information is valid.

-	Probability assertion: level of certainty, confidence in statistics/prediction.

## 6.2	Procedures for Data Collection

### 6.2.1	General

The Data Collection feature permits NWDAF to retrieve data from various sources (e.g. NF such as AMF, SMF, PCF, AF), as a basis of the computation of network analytics.

All available data encompass:

-	OAM global NF data,

-	behaviour data related to individual UEs or UE groups (e.g. UE reachability), and pre-computed metrics covering UE populations (e.g. number of UEs present in a geographical area), per spatial and temporal dimensions (e.g. per region for a period of time),

-	NF data available in the 5GC (e.g. NRF),

-	Data available in AF.

The NWDAF shall use at least one of the following services:

-	the Generic management services as defined in TS 28.532 [6], the Performance Management services as defined in TS 28.550 [7] or the Fault Supervision services as defined in TS 28.545 [9], offered by OAM in order to collect OAM global NF data.

-	the Exposure services offered by NFs in order to retrieve NF data and other non-OAM pre-computed metrics.

-	Other NF services in order to collect NF data (e.g. NRF)

The NWDAF shall obtain the proper information to perform data collection for a UE or group of UEs:

-	For an Analytics ID, NWDAF is configured with the corresponding NF Type(s) and/or event ID(s) and/or OAM measurement types.

-	NWDAF shall determine which NF instance(s) of the relevant NF type(s) are serving the UE or group of UEs, optionally taking into account the S-NSSAI(s) and area of interest as defined in clause 7.1.3, TS 23.501 [2].

-	NWDAF invokes Nnf_EventSubscribe services to collect data from the determined NF instance(s), and/or triggers the procedure in clause 6.2.3.2 to subscribe to OAM services to collect the OAM measurement.

The NWDAF performs data collection from an AF directly as defined in clause 6.2.2.2 or via NEF as defined in clause 6.2.2.3.

The NWDAF shall be able to discover the events supported by a NF.

Data collection procedures enables the NWDAF to efficiently obtain the appropriate data with the appropriate granularity.

When a request or subscription for statistics or predictions is received, the NWDAF may not possess the necessary data to perform the service.

-	Data on the monitoring period in the past matching the Analytics target period is necessary for the provision of statistics and predictions.

-	Data on longer monitoring periods in the past is necessary for model training.

Therefore, in order to optimize the service quality, the NWDAF may undertake the following actions:

-	The NWDAF may return a probability assertion as stated in clause 6.1.3 expressing the confidence in the analytics produced. With zero confidence, no analytics shall be returned. This confidence is likely to grow in the case of subscriptions.

-	The value of the confidence depends on the level or urgency expressed by the parameter "time when analytics information is needed" as listed in clause 6.1.3, the parameter "preferred level of accuracy of the analytics" as listed in clause 6.1.3, the availability of data. If no sufficient data is collected to provide an estimation for the requested level of accuracy before the time deadline, the service shall return a zero confidence. Otherwise, the NWDAF may wait until enough data is collected before providing a response or a first notification.

-	In order to be prepared for future requests on statistics from NFs/OAM, the NWDAF, upon operator configuration, may collect data on its own initiative, e.g. on samples of UEs (e.g. mobility), and retain the data collected in the data storage.

The volume and maximum duration of data storage is also subject of operator configuration.

The NWDAF may decide to reduce the amount of data collected to reduce signalling load, by either prioritizing requests received from analytics consumers, or reducing the extent (e.g. duration, scope) of data collection, or modifying the sampling ratios.

The NWDAF may skip data collection phase when the NWDAF already has enough information to provide requested analytics.

The data which NWDAF may collect is listed for each analytics in input data clause and is decided by the NWDAF.

NOTE:	NWDAF can skip data collection phase for some specific input data per the requested analytics e.g. when some of the data is already available at NWDAF for the requested analytics, or when NWDAF considers that some of the data is not needed at all to provide the requested analytics as per the analytics consumer request (e.g. based on preferred level of accuracy or based on the time when analytics are needed).

### 6.2.2	Data Collection from NFs

#### 6.2.2.1	General

The Data Collection from NFs is used by NWDAF to subscribe/unsubscribe at any NF to be notified for data on a set of events.

The Data Collection from NFs is based on the services of AMF, SMF, UDM, PCF, NRF and AF (possibly via NEF):

-	Event Exposure Service offered by each NF as defined in TS 23.502 [3] clause 4.15 and clause 5.2.

-	other NF services (e.g. Nnrf_NFDiscovery and Nnrf_NFManagement in NRF as defined in TS 23.502 [3] clause 4.17)

This data collection service is used directly in order to retrieve behaviour data for individual UEs or groups of UEs (e.g. UE reachability), and also to retrieve global UE information (e.g. Number of UEs present in a geographical area).

Table 6.2.2.1-1: Services consumed by NWDAF for data collection

| Service producer | Service | Reference in TS 23.502 [3] |
| --- | --- | --- |
| AMF | Namf_EventExposure | 5.2.2.3 |
| SMF | Nsmf_EventExposure | 5.2.8.3 |
| PCF | Npcf_EventExposure (for a group of UEs or any UE) Npcf_PolicyAuthorization_Subscribe (for a specific UE) | 5.2.5.7 |
| UDM | Nudm_EventExposure | 5.2.3.5 |
| NEF | Nnef_EventExposure | 5.2.6.2 |
| AF | Naf_EventExposure | 5.2.19.2 |
| NRF | Nnrf_NFDiscovery | 5.2.7.3 |
|  | Nnrf_NFManagement | 5.2.7.2 |

NOTE 1:	The present document specifies that NWDAF can collect some UPF input data for deriving analytics, but how NWDAF collects these UPF input data is not defined in this Release of the specification.

To retrieve data related to a specific UE, the NWDAF shall first determine which NF instances are serving this UE as stated in table 6.2.2.1-2 unless the NWDAF has already obtained this information due to recent operations related to this UE.

Table 6.2.2.1-2: NF Services consumed by NWDAF to determine which NF instances are serving a UE

| Type of NF instance (serving the UE) to determine | NF to be contacted by NWDAF | Service | Reference in TS 23.502 [3] |
| --- | --- | --- | --- |
| UDM | NRF | Nnrf_NFDiscovery | 5.2.7.3 |
| AMF | UDM | Nudm_UECM | 5.2.3.2 |
| SMF | UDM | Nudm_UECM | 5.2.3.2 |
| BSF | NRF | Nnrf_NFDiscovery | 5.2.7.3 |
| PCF | BSF | Nbsf_Management | 5.2.13.2 |
| NEF | NRF | Nnrf_NFDiscovery | 5.2.7.3 |

The UDM instance should be determined using NRF as described in clause 4.17.4 of TS 23.502 [3] and factors to determine as described in clause 6.3.8 of TS 23.501 [2].

The AMF, SMF instances should be determined using a request to UDM providing the SUPI or the group identity. To determine the SMF serving a PDU session, the NWDAF should in addition provide the DNN and S-NSSAI of this PDU Session; otherwise the NWDAF will obtain a list of possibly multiple SMFs (e.g. one per PDU session).

The BSF instance should be discovered using NRF thanks to optional request parameters (e.g. DNN list, IP domain list, IPv4 address range, IPv6 prefix range) as stated in clause 4.17.4 of TS 23.502 [3], or based on local configuration at the NWDAF.

The PCF instance serving UE PDU Session(s) should be determined using a request to BSF with the allocated UE address, DNN and S-NSSAI.

When NWDAF receives a request addressed to an Internal Group ID from a consumer, NWDAF may need to initiate data collection from several Network Functions, such as AMF, SMF, UDM, PCF, NEF/AF, etc. NWDAF may first discover the instances of the required Network Functions deployed in the network, e.g. by querying NRF.

For discovering the UDM, NWDAF can query the NRF with the Internal Group ID as the target of the query. For discovering AMF, SMF, PCF, NEF, and AF, NWDAF may need to discover all the instances in the network by using the Nnrf_NFDiscovery service.

NOTE 2:	It is assumed that all members of an Internal Group ID belong to the same UDM Group ID. NWDAF can select a UDM instance supporting the UDM Group ID of the Internal Group ID.

Then, if data needs to be collected from AMF, SMF, UDM, and PCF, NWDAF may initiate the data collection with the Internal Group ID as the target, e.g. subscribing to the event exposure in all the instances of a given type of network function. This subscription to all the instances of required source of event exposure handles, e.g. mobility of UEs across AMFs, or initiation of new PDU sessions with different allocated SMFs.

For collecting data from AMF and SMF, NWDAF may collect the data directly from AMF and/or SMF, or indirectly via UDM, according to TS 23.502 [3] clause 4.15.3.2.3.

The NEF instance that is serving a specific network slices and/or applications of a UE should be determined using NRF using optional request parameters as defined in clause 6.3.14 of TS 23.501 [2]

If NWDAF needs to collect data from an AF deployed outside the operator's domain, the NWDAF shall contact NEF with a SUPI or Internal Group ID as the target of the data collection. NEF is responsible for translation of SUPI to GPSI, or internal to external group identifiers, by querying UDM, prior to contacting the AF.

#### 6.2.2.2	Procedure for Data Collection from NFs

The procedure in Figure 6.2.2.2-1 is used by NWDAF to subscribe/unsubscribe at NFs in order to be notified for data collection on a related event (s), using Event Exposure Services as listed in Table 6.2.2.1-1.

Figure 6.2.2.2-1: Event Exposure Subscribe/unsubscribe for NFs

1.	The NWDAF subscribes to or cancels subscription for a (set of) Event ID(s) by invoking the Nnf_EventExposure_Subscribe / Nnf_EventExposure_Unsubscribe service operation.

NOTE 1:	The Event ID (s) are defined in TS 23.502 [3].

2.	If NWDAF subscribes to a (set of) Event ID(s), the NFs notify the NWDAF (e.g. with the event report) by invoking Nnf_EventExposure_Notify service operation.

NOTE 2:	The NWDAF can use the immediate reporting flag as defined in Table 4.15.1-1 of TS 23.502 [3] to meet the request-response model for data collection from NFs.

#### 6.2.2.3	Procedure for Data Collection from AF via NEF

The procedure in Figure 6.2.2.3-1 is used by NWDAF to collect information from AFs via the NEF.

NOTE 1:	In this release, AF registers its available data to NWDAF via OAM configuration at NEF.

The AF collectable data information includes: AF identification, AF service identification (e.g., endpoint information of Naf_EventExposure), available data to be collected per application (e.g., identified by Event ID(s)).

Figure 6.2.2.3-1: Data Collection from AF via NEF

1a.	After the registration of AF available data at the NEF, NEF generates an event exposure with new EventID to be associated with available data to be collected from AF. NEF invokes Nnrf_NFManagement_NFUpdate_request service operation to update its registration information (i.e., NEF Profile) including the generated Event IDs, and associated AF identification, Application ID(s).

1b.	NRF stores the received NEF registration information including available data to be collected from AF.

1c.	NRF sends Nnrf_NFManagement_NFUpdate_response message to NEF.

1d.	When NWDAF needs to discovery the available data from AFs and the appropriated NEF to collect this data, NWDAF invokes Nnrf_NFDiscovery_Request_request service operation using as parameter the NEF NF Type, a list of Event ID(s), and optionally AF identification, application ID.

1e.	NRF matches the requested query for available data in AFs with the registered NEF Profiles and sends this information via Nnrf_NFDiscovery_Request_response message to NWDAF.

NOTE 2:	After the registration and discovery procedure described in step 1, NWDAF identifies the available data per AF per application and the proper NEF to collect such data.

2.	The NWDAF subscribes to or cancels subscription to data in AF via NEF by invoking the Nnef_EventExposure_Subscribe or Nnef_EventExposure_Unsubscribe service operation. If the event subscription is authorized by the NEF, the NEF records the association of the event trigger and the NWDAF identity.

NOTE 3:	User consent for retrieving user data in AF via NEF is not specified in this Release.

3.	Based on the request from the NWDAF, the NEF subscribes to or cancels subscription to data in AF by invoking the Naf_EventExposure_Subscribe/ Naf_EventExposure_Unsubscribe service operation.

4.	If the NEF subscribes to data in AF, the AF notifies the NEF with the data by invoking Naf_EventExposure_Notify service operation.

5.	If the NEF receives the notification from the AF, the NEF notifies the NWDAF with the data by invoking Nnef_EventExposure_Notify service operation.

#### 6.2.2.4	Procedure for Data Collection from NRF

The NWDAF may use NRF services and Network Function service framework procedures as defined in TS 23.502 [3] clause 5.2.7 and clause 4.17:

-	NF/NF service discovery procedures (in TS 23.502 [3] clause 4.17.4) and Nnrf_NFDiscovery service (in TS 23.502 [3] clause 5.2.7.3) in order to dynamically discover the NF instances and services of the 5GC. Such discovery may be performed on a periodic basis, or under specific circumstances.

-	NF/NF service status subscribe/notify procedures (in TS 23.502 [3] clause 4.17.7) and Nnrf_NFManagement service (in TS 23.502 [3] clause 5.2.7.2) in order to be notified about the change of status of an NF. The service operations for obtaining status information are NFStatusSubscribe and NFStatusNotify, from the Nnrf_NFManagement service.

The information provided by the NRF to the NWDAF with the Nnrf_NFDiscovery_Request and the Nnrf_NFManagement_NFStatusNotify operations are the NFProfile and the NFService. Such information can be used to set-up and maintain a consistent network map for data collection and also, depending on use cases, in order to perform estimations:

-	NF profile: NF capacity, NF load information, NF status, NF recovery time, NF locality;

-	NF service: service capacity, service load information, service status, service recovery time.

#### 6.2.2.5	Usage of Exposure framework by the NWDAF for Data Collection

The NWDAF shall subscribe (and unsubscribe) to the Event exposure service from NF(s) reusing the framework defined in TS 23.502 [3] clause 4.15. This framework supports the possibility for the NWDAF to indicate / request:

-	Events-ID: one or multiple Event ID(s) defined in TS 23.502 [3] clause 4.15.1

-	Target of Event Reporting defined in TS 23.502 [3] clause 4.15.1: the objects targeted by the Events. Within a subscription, all Event ID(s) are associated with the same target of event reporting. In the case of NWDAF, the objects can be UE(s), UE group(s), any UE.

-	Event Filter Information defined in TS 23.502 [3] clause 4.15.1. This provides Event Parameter Types and Event Parameter Value(s) to be matched against.

-	A Notification Target Address and a Notification Correlation ID as defined in TS 23.502 [3] clause 4.15.1, allowing the NWDAF to correlate notifications received from the NF with this subscription.

-	Event Reporting Information described in TS 23.502 [3] Table 4.15.1-1.

-	Expiry time as defined in TS 23.502 [3] clause 4.15.1.

The notifications from NFs/AFs contain on top of the Event being reported (and of dedicated information being reported for this event):

-	the Notification Correlation Information provided by the NWDAF in its request,

-	(when applicable to the event) the Target Id e.g. UE ID (SUPI and if available GPSI), and

-	a time stamp.

### 6.2.3	Data Collection from OAM

#### 6.2.3.1	General

The NWDAF may collect relevant management data from the services in the OAM as configured by the PLMN operator.

‐	NG RAN or 5GC performance measurements as defined in TS 28.552 [8].

‐	5G End to end KPIs as defined in TS 28.554 [10].

NWDAF shall use the following services to have access to the information provided by OAM:

-	Generic performance assurance and fault supervision management services as defined in TS 28.532 [6].

‐	PM (Performance Management) services as defined in TS 28.550 [7].

‐	FS (Fault Supervision) services defined in TS 28.545 [9].

NWDAF can be configured to invoke the existing OAM services to retrieve the management data that are relevant for analytics generation, which may include NF resources usage information (e.g. usage of virtual resources assigned to NF) and NF resource configuration information (e.g. life cycle changes of NF resource configurations).

OAM perform the required configuration in order to provide the information requested by NWDAF subscription and perform the tasks, e.g. data collection, data processing, associated with the subscribed request from NWDAF.

Another usage of OAM services is when the target of data collection is a specific UE, via MDT based retrieval of information.

#### 6.2.3.2	Procedure for data collection from OAM

The interactions between NWDAF and OAM for data collection are illustrated in Figure 6.2.3.2-1. The data collected depends on the use cases. This figure is an abstraction of the OAM performance data file report management service that is defined TS 28.532 [6]. The actual OAM services and reporting mechanisms that NWDAF may use are specified in TS 28.532 [6], TS 28.550 [7] or TS 28.545 [9].

The flow below assumes the NWDAF is configured on how to subscribe to the relevant OAM services.

OAM shall setup the required mechanisms to guarantee the continuous data collection requested by NWDAF.

Figure 6.2.3.2-1: Data collection from OAM performance data file report management service

1.	(Clause 7.1.1.3.2, TS 28.532 [6]), Subscribe (Input): NWDAF subscribes to the notification(s) related to the services provided by the management service producer.

2.	(Clause 7.1.1.3.3, TS 28.532 [6]), Subscribe (Output): management service producer responses to NWDAF if the subscription is success or not.

3.	Data processing: management service producer prepares the data.

4.	(Clause 7.1.1.1, TS 28.532 [6]), Notification (notifyFileReady): management service producer notifies the data file is ready.

As the final step, NWDAF fetches data by using FTP (not specified in 3GPP, based on vendor implementation).

NOTE:	The call flow in Figure 6.2.3.2-1 only shows a subscribe/notify model for the simplicity, however both request-response and subscription-notification models are supported.

### 6.2.4	Correlation between network data and service data

The Correlation information per each NF is to help NWDAF correlate data from different NFs are defined in Table 6.2.4-1, which is subject to all the network data analytics.

NOTE:	For simplicity, the correlation information is not listed in the input data per network data analytics.

Table 6.2.4-1: Correlation Information per NF

| Correlation Information | Description |
| --- | --- |
| Time Interval and IP address 5-tuple | To correlate the data from AF and from UPF. |
| Time Interval and AN Tunnel Info (Clause 9.3.2.2, TS 38.413 [16]) | To correlate the UPF data and OAM data which are reported by the RAN (e.g. Reference Signal Received Power or Reference Signal Received Quality as defined in Table 6.4.2-3). |
| Time Interval and UE IP address | To correlate the data from UPF and SMF. |
| SUPI | To correlate data from SMF and AMF. |
| SUPI, DNN, S-NSSAI or UE IP address | To correlate data from SMF and PCF. |
| Time Interval, RAN UE NGAP ID (Clause 9.3.3.2, TS 38.413 [16]) and Global RAN Node ID | To correlate the AMF data and OAM data reported by the RAN (e.g. Reference Signal Received Power or Reference Signal Received Quality as defined in Table 6.4.2-3). |

## 6.3	Slice load level related network data analytics

### 6.3.1	General

The NWDAF provides slice load level information to an NF on a network slice instance level. The NWDAF is not required to be aware of the current subscribers using the slice. The NWDAF notifies slice specific network status analytics information to the NFs that are subscribed to it. An NF may collect directly slice specific network status analytics information from NWDAF. This information is not subscriber specific.

The NWDAF services as defined in the clause 7.2 and clause 7.3 are used to expose slice load level analytics from the NWDAF to the consumer NF (e.g. PCF or NSSF).

The following Analytics ID is used for the slice load level related network data analytics:

-	Load level information

The following Analytics Filters can be included by the consumer in the related Nnwdaf_AnalyticsSubscription_Subscribe and Nnwdaf_AnalyticsInfo_Request service operation:

-	S-NSSAI and the associated NSI ID.

-	Load Level Threshold value.

### 6.3.2	Void

### 6.3.2A	Input data

There is no input data specification for support of slice load level analytics in this Release of the specification.

### 6.3.3	Void

### 6.3.3A	Output analytics

The NWDAF reports when the load level of the Network Slice Instance, indicated by the S-NSSAI and the associated NSI ID in the Analytics Filter, crosses the threshold provided in the analytics subscription; if no threshold is provided in the subscription, the reporting (Notify operation) is assumed to be periodic.

## 6.4	Observed Service experience related network data analytics

### 6.4.1	General

This clause specifies how NWDAF can provide Observed Service Experience (i.e. average observed Service MoS) analytics, in the form of statistics or predictions, to a service consumer.

The Observed Service Experience analytics may provide one or both of the following:

-	Service Experience for a Network Slice: Service Experience for UEs (for a UE or a group of or any UE) for a given Application or a set of Applications or any Application (i.e. all Applications) in a Network Slice;

-	Service Experience for an Application: Service Experience (i.e. for a UE or a group of UEs or any UE) in an Application.

Therefore, Observed Service experience may be provided individually per UE or group of UEs, or globally, averaged per Application or averaged across a set of Applications on a Network Slice.

The service consumer may be an NF (e.g. PCF), or the OAM.

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics Id set to "Service Experience";

-	The Target of Analytics Reporting: one or more SUPI(s) or Internal Group Identifier(s), or "any UE";

-	Analytics Filter Information as defined in Table 6.4.1-1;

Table 6.4.1-1: Analytics Filter Information related to the observed service experience

| Information | Description |
| --- | --- |
| Application ID (1..n) (NOTE 1) | The identification of the application(s) for which the analytics information is subscribed or requested. |
| S-NSSAI (NOTE 2) | Identifies the Network Slice for which analytics information is subscribed or requested. |
| Area of Interest | Identifies the Area (i.e. set of TAIs), as defined in TS 23.501 [2] where the analytics information is subscribed or requested. |
| Media/application bandwidth | Identifies the Media/application bandwidth requirement of the application. |
| DNN | DNN to access the application. |
| DNAI | Identifier of a user plane access to one or more DN(s) where applications are deployed as defined in TS 23.501 [2] |
| NOTE 1: If no Application ID is provided, the Analytics Filter information applies to any application (i.e. all applications) in the Network Slice. NOTE 2: The S-NSSAI is mandatory if the NWDAF Service Consumer subscribes or requests the Service Experience in a Network Slice. |  |

-	An Analytics target period that indicates the time window for which the statistics or predictions are requested;

-	In a subscription, the Notification Correlation Id and the Notification Target Address.

The NWDAF shall notify the result of the analytics to the consumer as specified in clause 6.4.3.

NWDAF collects the network data from AF (directly or via NEF) and from other 5GC NF(s) in order to calculate and provides statistics and predictions on the observed service experience to a consumer NF or to OAM.

### 6.4.2	Input Data

The service data collected from the AF, the network data from other 5GC NFs and the network data from OAM for observed service experience are defined in Table 6.4.2-1, Table 6.4.2-2 and Table 6.4.2-3, respectively.

Table 6.4.2-1: Service Data from AF related to the observed service experience

| Information | Source | Description |
| --- | --- | --- |
| Application ID | AF | To identify the service and support analytics per type of service (the desired level of service) |
| IP filter information | AF | Identify a service flow of the UE for the application |
| Locations of Application | AF/NEF | Locations of application represented by a list of DNAI(s). The NEF may map the AF-Service-Identifier information to a list of DNAI(s) when the DNAI(s) being used by the application are statically defined. |
| Service Experience | AF | Refers to the QoE per service flow as established in the SLA and during on boarding. It can be either e.g. MOS or video MOS as specified in ITU-T P.1203.3 [11] or a customized MOS |
| Timestamp | AF | A time stamp associated to the Service Experience provided by the AF, mandatory if the Service Experience is provided by the ASP. |

NWDAF subscribes to the service data from AF in the Table 6.4.2-1 either directly by invoking Naf_EventExposure_Subscribe service (Event ID = Service Data, Event Filter information = Application ID) as defined in TS 23.502 [3], or via NEF by invoking Nnef_EventExposure_Subscribe service.

Table 6.4.2-2: QoS flow level Network Data from 5GC NF related to the QoS profile assigned for a particular service (identified by an Application Id or IP filter information)

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | 5GC NF | A time stamp associated with the collected information. |
| Location | AMF | The UE location information. |
| DNN | SMF | DNN for the PDU Session which contains the QoS flow |
| S-NSSAI | SMF | S-NSSAI for the PDU Session which contains the QoS flow |
| Application ID | SMF | Used by NWDAF to identify the application service provider and application for the QoS flow |
| IP filter information | SMF | Provided by the SMF, which is used by NWDAF to identify the service data flow for policy control and/or differentiated charging for the QoS flow |
| QFI | SMF | QoS Flow Identifier |
| QoS flow Bit Rate | UPF | The observed bit rate for UL direction; and The observed bit rate for DL direction |
| QoS flow Packet Delay | UPF | The observed Packet delay for UL direction; and The observed Packet delay for the DL direction |
| Packet transmission | UPF | The observed number of packet transmission |
| Packet retransmission | UPF | The observed number of packet retransmission |

NOTE:	How NWDAF collects QoS flow Bit Rate, QoS flow Packet Delay, Packet transmission and Packet retransmission information is not defined in this Release of the specification.

NWDAF subscribes to the network data from 5GC NF(s) in the Table 6.4.2-2 by invoking Nnf_EventExposure_Subscribe service operation. with the following Event IDs as input parameters:

-	AMF Source: Namf_EventExposure_Subscribe (Event ID = Location Changes).

-	SMF Source: Nsmf_EventExposure_Subscribe (Event ID = QFI allocation).

Table 6.4.2-3: UE level Network Data from OAM related to the QoS profile

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | OAM | A time stamp associated with the collected information. |
| Reference Signal Received Power | OAM | The per UE measurement of the received power level in a network cell, including SS-RSRP, CSI-RSRP as specified in clause 5.5 of TS 38.331 [14] and E-UTRA RSRP as specified in clause 5.5.5 of TS 36.331 [15] |
| Reference Signal Received Quality | OAM | The per UE measurement of the received quality in a network cell, including SS-RSRQ, CSI-RSRQ as specified in clause 5.5 of TS 38.331 [14] and E-UTRA RSRQ as specified in clause 5.5.5 of TS 36.331 [15] |
| Signal-to-noise and interference ratio | OAM | The per UE measurement of the received signal to noise and interference ratio in a network cell, including SS-SINR, CSI-SINR, E-UTRA RS-SINR, as specified in clause 5.1 of TS 38.215 [12] |

NWDAF subscribes the network data from OAM in the Table 6.4.2-3 by using the services provided by OAM as described in clause 6.2.3.

The Event Filter for the service data collection from AF is defined in Table 6.4.2-4.

Table 6.4.2-4: Event Filter Information related to the service data from AF

| Information | Presence | Description |
| --- | --- | --- |
| Application ID (1..n) | Conditional (NOTE 1) | An identification of the application or a set of identifications of the applications. |
| Area of Interest | Conditional (NOTE 2) | Area of Interest which restricts the area in focus |
| NOTE 1: If no Application ID is provided, the Event Filter information applies to any Application (i.e. all Applications). NOTE 2: Mandatory if Target Of Event Reporting is set to "any UE", optional otherwise. |  |  |

The Event Filter for the service data collection from SMF and AMF are defined in TS 23.502 [3].

The timestamps are provided by each NF to allow correlation of QoS and traffic KPIs. The clock reference is able to know the accuracy of the time and correlate the time series of the data retrieved from each NF.

### 6.4.3	Output Analytics

The NWDAF services as defined in the clause 7.2 and 7.3 are used to expose the analytics.

-	Service Experience statistics information is defined in Table 6.4.3-1.

-	Service Experience predictions information is defined in Table 6.4.3-2.

Table 6.4.3-1: Service Experience statistics

| Information | Description |
| --- | --- |
| S-NSSAI | Identifies the Network Slice for which analytics information is provided. |
| ServiceExperience (1..n) | List of observed service experience information for each Application. |
| > Application ID | Identification of the application. |
| > Service Experience Type | Type of Service Experience analytics, e.g. on voice, video, other. |
| > Service Experience | Service Experience over the Analytics target period (average, variance). |
| > SUPI list (1..n) | List of SUPI(s) for each application, applicable only to detailed Service Experience. |
| > Ratio | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity | Area where the estimated Service Experience applies. If Area of Interest information was provided in the request or subscription, spatial validity may be a subset of the requested Area of Interest. |
| > Validity period | Validity period as defined in clause 6.1.3. |
| Slice service experience | Service experience across applications on a Network Slice over the Analytics target period (average, variance). |

Table 6.4.3-2: Service Experience predictions

| Information | Description |
| --- | --- |
| S-NSSAI | Identifies the Network Slice for which analytics information is provided. |
| ServiceExperience (1..n) | List of predicted service experience information for each Application. |
| > Application ID | Identification of the application. |
| > Service Experience Type | Type of Service Experience analytics, e.g. on voice, video, other. |
| > Service Experience | Service Experience over the Analytics target period (average, variance). |
| > SUPI list (1..n) | List of SUPI(s) for each application, applicable only to detailed Service Experience. |
| > Ratio | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity | Area, where the estimated Service Experience applies. If Area of Interest information was provided in the request or subscription, spatial validity may be a subset of the requested Areas of Interest. |
| > Validity period | Validity period as defined in clause 6.1.3. |
| > Probability assertion | Confidence of this prediction. |
| Slice service experience | Service experience across applications on a Network Slice over the Analytics target period (average, variance). |

The number of Service Experiences and SUPIs is limited by the maximum number of results provided as input parameter.

### 6.4.4	Procedures to request Service Experience for an Application

Figure 6.4.4-1: Procedure for NWDAF providing Service Experience for an Application

This procedure allows the consumer to request Analytics ID "Service Experience" for a particular Application. The consumer includes both the Application ID for which the Service Experience is requested and indicates that the Target of Analytics Reporting is "any UE".  At the same time, for an Application ID, a set of initial QoS parameter combinations per service experience window (e.g. one is for 3<Service MOS<4 and another is for 4<Service MOS<5) is defined in PCF (e.g. by configuration of operator policies) that may be updated based on the Service Experience reported by NWDAF.

1.	Consumer NF sends an Analytics request/subscribe (Analytics ID = Service Experience, Target of Analytics Reporting = any UE, Analytics Filter information = (Application ID, Analytics target period S-NSSAI, DNN, Area of Interest)) to NWDAF by invoking a Nnwdaf_AnalyticsSubscription_Subscribe.

2a. NWDAF subscribes the service data from AF in the Table 6.4.2-1 by invoking Nnef_EventExposure_Subscribe or Naf_EventExposure_Subscribe service (Event ID = Service Data, Event Filter information = (Application ID, Area of Interest), Target of Event Reporting = Any UE) as defined TS 23.502 [3].

NOTE 1:	In the case of external AF, NEF translates the requested Area of Interest into a list of geographic zone identifier(s) as described in clause 5.6.7.1 of TS 23.501 [2].

2b.	NWDAF subscribes the network data from 5GC NF(s) in the Table 6.4.2-2 by invoking Nnf_EventExposure_Subscribe service operation.

2c. With these data, the NWDAF estimates the Service experience for the application.

NOTE 2:	QoE measurements from the applications are based on outcome of the ongoing SA5 Rel-16 WID  "Management of QoE measurement collection" which addresses how to collect the QoE measurements from the applications in the UE.

3.	The NWDAF provides the data analytics, i.e. the observed Service Experience (which can be a range of values) to the consumer NF by means of Nnwdaf_AnalyticsSubscription_Notify, indicating how well the used QoS Parameters satisfy the Service MoS agreed between the MNO and the end user or between the MNO and the external ASP.

NOTE 3:	The call flow only shows a request-response model for the interaction of NWDAF and consumer NF for simplicity instead of both request-response model and subscription-notification model.

If the consumer NF is PCF and it determines that the application SLA is not satisfied, it may take into account the Observed Service Experience and the operator policies including SLA and required Service Experience (which can be a range of values) to determine new QoS parameters to be applied for the service, as defined in clause 6.1.1.3 and clause 6.2.1.2, TS 23.503 [4].

NOTE 4:	The non-real time data information from AF includes the service experience data (see Table 6.4.2-1), which indicates the service quality during the service lifetime.

### 6.4.5	Procedures to request Service Experience for a Network Slice

Figure 6.4.5-1: Procedure for NWDAF providing Service Experience for a UE or a group of UEs in a Network Slice

This procedure is similar to the procedure in clause 6.4.4, with the following differences. The consumer needs to request the Analytics ID "Service Experience" for all UEs or a group of UEs on a Network Slice, identified by an S-NSSAI. In addition, service experience data may need to be collected from multiple Applications. If each Application is hosted in a separate AF, NWDAF subscribes the service data in the Table 6.4.2-1 from the different AFs by invoking Nnef_EventExposure_Subscribe or Naf_EventExposure_Subscribe services for each Application (Event ID = Service Data, Event Filter information = (Application ID, Area of Interest), Target of Event Reporting = Any UE) as defined TS 23.502 [3]. Figure 6.4.5-1 shows an example procedure with two AFs.

If one AF provides the service experience data of multiple Applications, the set of Application IDs is provided by NWDAF to the AF with the Naf_EventExposure_Subscribe service operation, as defined TS 23.502 [3].

NOTE:	The call flow only shows a request-response model for the interaction of NWDAF and consumer NF for simplicity instead of both request-response model and subscription-notification model.

The Observed Service Experience for a Network Slice when consumed by OAM could be used as described in Annex H of TS 28.550 [7].

## 6.5	NF load analytics

### 6.5.1	General

The clause 6.5 describes how NWDAF can provide NF load analytics, in the form of statistics or predictions or both, to another NF.

The service consumer may be an NF, or the OAM.

The consumer of these analytics shall indicate in the request:

-	Analytics ID set to "NF load information";

-	The Target of Analytics Reporting: an optional SUPI;

-	Analytics Filter Information:

-	optional S-NSSAI;

-	an optional list of NF Instance IDs, NF Set IDs, or NF types;

-	Optional Reporting Threshold; the Reporting Threshold is unique for all NFs matching the above Analytics Filter and the reporting applies when the conditions are met for at least one of these NFs;

-	An Analytics target period indicates the time period over which the statistics or predictions are requested;

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

The NWDAF shall notify the result of the analytics to the consumer as indicated in clause 6.5.3.

If a list of the NF Instance IDs (or respectively of NF Set IDs) is provided, the NWDAF shall provide the analytics for each designated NF instance (or respectively for each NF instance belonging to each designated NF Set). In such case the Target of Analytics Reporting should be ignored.

Otherwise, if a SUPI is provided, the NWDAF shall use the SUPI to determine which NF instances (AMF and SMF) are serving this specific UE, filter them according to the provided S-NSSAI and NF types using data collected from NRF or OAM, and provide analytics for these NF instances.

NOTE:	Only NF instances of type AMF and SMF can be determined using a SUPI.

### 6.5.2	Input data

For the purpose of NF load analytics, the NWDAF may collect the information as listed in Table 6.5.2-1 for the relevant NF instance(s).

Table 6.5.2-1: Data collected by NWDAF for NF load analytics

| Information | Source | Description |
| --- | --- | --- |
| NF load | NRF | The load of specific NF instance(s) in their NF profile as defined per TS 29.510 [18]. |
| NF status | NRF | The status of a specific NF instance(s) (registered, suspended, undiscoverable) as defined per TS 29.510 [18]. |
| NF resource usage | OAM | The usage of assigned virtual resources currently in use for specific NF instance(s) (mean usage of virtual CPU, memory, disk) as defined in TS 28.552 [8] clause 5.7. |
| NF resource configuration | OAM | The life cycle changes of specific NF resources (e.g., NF operational or interrupted during virtual/physical resources reconfiguration) as defined in TS 28.533 [19], clause 5.2. |

NOTE 1:	The OAM information can be used as a complement to NRF information for some or all of the following aspects: resources utilization, NRF information correlation, and alternative source of information if NRF information on load is not available.

NOTE 2:	NWDAF can request NRF for data related to NF instances, as described in TS 29.510 [18].

NOTE 3:	NWDAF can correlate the NF resources configuration with NF resource usage for generating the analytics output.

If target NF type is UPF, the NWDAF may collect the information as listed in Table 6.5.2-2, in addition to information listed in Table 6.5.2-1.

Table 6.5.2-2: Collecting Data for analysing UPF load analytics

| Information | Source | Description |
| --- | --- | --- |
| Traffic usage report | UPF | Report of user plane traffic in the UPF for the accumulated usage of network resources (see TS 29.244 [17]) |

NOTE 4:	How NWDAF collects information in table 6.5.2-2 is not defined in this Release of the specification.

### 6.5.3	Output analytics

The NWDAF services as defined in the clause 7.2 and 7.3 are used to expose the analytics. NF load statistics information are defined in Table 6.5.3-1. NF load predictions information are defined in Table 6.5.3-2.

Table 6.5.3-1: NF load statistics

| Information | Description |
| --- | --- |
| List of resource status (1..n) | List of observed load information for each NF instance along with the corresponding NF id / NF Set ID (as applicable) |
| > NF type | Type of the NF instance |
| > NF instance ID | Identification of the NF instance |
| > NF status | The availability status of the NF on the Analytics target period, expressed as a percentage of time per status value (registered, suspended, undiscoverable) |
| > NF resource usage | The average usage of assigned resources (CPU, memory, disk) |
| > NF load | The average load of the NF instance over the Analytics target period |
| > NF peak load (optional) | The maximum load of the NF instance over the Analytics target period |

Table 6.5.3-2: NF load predictions

| Information | Description |
| --- | --- |
| List of resource status (1..n) | List of predicted load information for each NF instance along with the corresponding NF id / NF Set ID (as applicable) |
| > NF type | Type of the NF instance |
| > NF instance ID | Identification of the NF instance |
| > NF status | The availability status of the NF on the Analytics target period, expressed as a percentage of time per status value (registered, suspended, undiscoverable) |
| > NF resource usage | The average usage of assigned resources (CPU, memory, disk) |
| > NF load | The average load of the NF instance over the Analytics target period |
| > NF peak load (optional) | The maximum load of the NF instance over the Analytics target period |
| > Confidence | Confidence of this prediction |

NOTE:	The variations on per-instance NF load and resource usage could be influenced by the number of running NF instances in addition to the load itself.

The predictions are provided with a Validity Period, as defined in clause 6.1.3.

### 6.5.4	Procedures

The procedure depicted in Figure 6.5.4-1 allows a consumer NF to request analytics to NWDAF for NF load of various NF instances as defined in 6.5.1.

Figure 6.5.4-1: NF load analytics provided by NWDAF

1.	The NF sends a request to the NWDAF for analytics for NF load for a specific NF, using either the Nnwdaf_AnalyticsInfo or Nnwdaf_AnalyticsSubscription service. The Analytics ID is set to NF load information, the target for analytics and the analytics filter are set according to clause 6.5.1. The NF can request statistics or predictions or both and can provide a time window.

2-5.	If the request is authorized, and in order to provide the requested analytics, the NWDAF may need for each NF targeted instance to subscribe to OAM services to retrieve the target NF resource usage and NF resources configuration following steps captured in clause 6.2.3.2 for data collection from OAM. Steps 2-5 may be skipped when e.g. the NWDAF already has the requested analytics.

NOTE:	The call flow only shows a subscription/notification model for the simplicity, however both request-response and subscription-notification models should be supported.

6a.	The NWDAF subscribes to changes on the load and status of NF instances registered in NRF and identified by their NF id from NRF using Nnrf_NFManagement_NFStatusSubscribe service operation for each NF instance.

6b. NRF notifies NWDAF of changes on the load and status of the requested NF instances by using Nnrf_NFManagement_NFStatusNotify service operation.

7.	The NWDAF derives requested analytics.

8.	The NWDAF provide requested NF load analytics to the NF along with the corresponding Validity Period, using either the Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Subscribe response, depending on the service used in step 1.

9-11. If at step 1 the NF has subscribed to receive continuous reporting of NF load analytics, the NWDAF may generate new analytics and, when relevant according to the Analytics target period and Reporting Threshold, provide them along with the corresponding Validity Period to the NF upon reception of notification of new NF load information from OAM.

## 6.6	Network Performance Analytics

### 6.6.1	General

With Network Performance Analytics, NWDAF provides either statistics or predictions on the load, communication and mobility performance in an Area of Interest; in addition, NWDAF it may provide statistics or predictions on the number of UEs that are located in that Area of Interest.

The service consumer may be an NF (e.g. PCF, NEF, AF), or the OAM.

The consumer of these analytics may indicate in the request:

-	Analytics ID set to "Network Performance";

-	Target of Analytics Reporting containing either a UE, or Internal Group Identifier that refers to the group for which the analytics on the number of UEs that are located in the Area of Interest at the time indicated in the Analytics target period is requested or any UE;

-	Analytics Filter Information containing:

-	Optionally, maximum number of results;

-	Area of Interest (list of TA or Cells) which restricts the area in focus (mandatory if Target Of Event Reporting is set to "any UE", optional otherwise);

-	Optionally, the subset of analytics that are requested among those specified in clause 6.6.3;

-	Optionally, Reporting Thresholds, which apply only for subscriptions and indicate conditions on the level to be reached for respective analytics information (see clause 6.6.3) in order to be notified by the NWDAF;

-	An Analytics target period indicates the time period over which the statistics or prediction are requested; and

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

The NWDAF notifies the result of the analytics to the consumer as indicated in clause 6.6.3.

### 6.6.2	Input Data

The NWDAF collects Load and Performance information in an Area of Interest from the sources listed in Table 6.6.2-1 and number of UEs within Area of Interest from the sources listed in Table 6.6.2-2.

Table 6.6.2-1:  Load and Performance information collected by NWDAF

| Load information | Source | Description |
| --- | --- | --- |
| Status, load and performance information | OAM | Statistics on RAN status (up/down), load (i.e. Radio Resource Utilization) and performance per Cell Id in the Area of Interest as defined in TS 28.552 [8]. |
| NF Load information | NRF | Load per NF |

Table 6.6.2-2:  Number of UEs in Area of Interest information collected by NWDAF

| Number of UEs information | Source | Description |
| --- | --- | --- |
| Number of UEs | AMF | Number of UEs in an Area of Interest |

The NWDAF shall be able to collect UE mobility information as stated in clause 6.7.2.2.

### 6.6.3	Output Analytics

The NWDAF shall be able to provide both statistics and predictions on Network Performance.

Network performance statistics are defined in Table 6.6.3-1.

Table 6.6.3-1: Network performance statistics

| Information | Description |
| --- | --- |
| List of network performance information (1..max) | Observed statistics during the Analytics target period |
| > Area subset | TA or Cell ID within the requested area of interest |
| > gNB status information | Ratio of gNBs that have been up and running during the entire Analytics target period in the area subset |
| > gNB resource usage | Average usage of assigned resources (CPU, memory, disk) |
| > Number of UEs | Number of UEs observed in the area subset |
| > Communication performance | Ratio of successful setup of PDU Sessions |
| > Mobility performance | Ratio of successful handover |

Network performance predictions are defined in Table 6.6.3-2.

Table 6.6.3-2: Network performance predictions

| Information | Description |
| --- | --- |
| List of network performance information (1..max) | Predicted analytics during the Analytics target period |
| > Area subset | TA or Cell ID within the requested area |
| > gNB status information | Ratio of gNBs that will be up and running during the entire Analytics target period in the area subset |
| > gNB resource usage | Usage of assigned resources (CPU, memory, disk) (average, peak) |
| > Number of UEs | Number of UEs predicted in the area subset |
| > Communication performance | Ratio of successful setup of PDU Sessions |
| > Mobility performance | Ratio of successful handover |
| > Confidence | Confidence of this prediction |

NOTE 1:	The predictions are provided with a Validity Period, as defined in clause 6.1.3.

NOTE 2:	The analytics on number of UEs are related to the information retrieved from the AMFs.

The NWDAF provides Network Performance Analytics to a consumer that includes the load in the area of interest and the number of UEs located in the area of interest or both at the time requested by the consumer in the Analytics target period:

-	Analytics ID set to "Network Performance".

-	Notification Target Address including the address of the consumer.

-	Notification Correlation Id, for the consumer to correlate notifications from NWDAF if subscription applies.

-	Analytics specific parameters including the load in the Area of Interest and the number of UEs in the Area of Interest, at the time indicated in the Analytics target period.

### 6.6.4	Procedures

Figure 6.6.4-1: Procedure for subscription to network performance analytics

1.	The NF sends Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request (Analytics ID="Network Performance", Target of Analytics Reporting, Analytics Filter="Area of Interest", "Reporting Thresholds" and Analytics target Period(s)) to the NWDAF.

2a-2d.	The NWDAF discovers from NRF the AMF(s) belonging to the AMF Region(s) that include(s) the Area of Interest and subscribes to NF load and status information from NRF about these AMF(s).

3a-3b.	The NWDAF subscribes to OAM services to get the status and load information and the resource usage on the Area of Interest, following the procedure captured in Clause 6.2.3.2.

Editor's note:	The exact SA WG5 service, granularity (NF, eNB) and measurement or KPI that are used for load.

4a-4b.	The NWDAF collects the number of UEs located in the Area of Interest from AMF using Namf_EventExposure_Subscribe service, including the Target of Event Reporting provided as an input parameter (i.e. any UE or Internal Group Identifier).

5.	The NWDAF derives the requested analytics.

6.	The NWDAF sends Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request response (one or more instances of the n-tuple (Load in Area of Interest and number of UEs, timestamp) SubscriptionCorrelationId, Probability of assertion).

7-8. A change of network performance information, i.e. change in the load in the area of interest at the observed period, is detected by OAM, or a change in the NF load information is reported by NRF, and is notified to NWDAF.

9.	The NWDAF derives new analytics taking into account the most recent data collected.

10.	When relevant according to the Analytics target period and Reporting Thresholds, the NWDAF provides a notification using Nnwdaf_AnalyticsSubscription_Notify (one or more instances of the n-tuple (Load in Area of Interest and number of UEs, timestamp) SubscriptionCorrelationId, Probability of assertion.

## 6.7	UE related analytics

### 6.7.1	General

This clause specifies the UE related analytics which can be provided by NWDAF:

-	UE mobility analytics;

-	UE communication analytics;

-	Expected UE behavioural parameters related network data analytics; and

-	Abnormal behaviour related network data analytics.

The NWDAF service consumer may request for these analytics separately, or in a combined way. As an example, an NWDAF service consumer may learn from the NWDAF the expected UE behaviour parameters as defined in clause 4.15.6.3, TS 23.502 [3] for a group of UEs or a specific UE, by requesting analytics for both UE mobility (see clause 6.7.2) and for UE communication (see clause 6.7.3).

NOTE:	Possible uses of such analytics is for the AMF to learn about expected UE behaviour to derive appropriate MICO mode configuration, or for an AF to learn about expected UE behaviour to further provision 5GC with appropriate UE parameters.

### 6.7.2	UE mobility analytics

#### 6.7.2.1	General

NWDAF supporting UE mobility statistics or predictions shall be able to collect UE mobility related information from NF, OAM, and to perform data analytics to provide UE mobility statistics or predictions.

The consumer of these analytics may indicate in the request:

-	The Target of Analytics Reporting which is a single UE or a group of UEs.

-	Analytics Filter Information containing optional list of TA or Cells, optional maximum number of results

-	An Analytics target period indicates the time period over which the statistics or predictions are requested.

-	Preferred level of accuracy of the analytics (low/high).

#### 6.7.2.2	Input Data

The NWDAF supporting data analytics on UE mobility shall be able to collect UE mobility information from OAM, 5GC and AFs. The detailed information collected by the NWDAF could be MDT data from OAM, network data from 5GC and/or service data from AFs:

-	UE mobility information from OAM is UE location carried in MDT data;

-	Network data related to UE mobility from 5GC is UE location information as defined in the Table 6.7.2.2-1;

Table 6.7.2.2-1: UE Mobility information collected from 5GC

| Information | Source | Description |
| --- | --- | --- |
| UE ID | AMF | SUPI |
| UE locations (1..max) | AMF | UE positions |
| >UE location |  | TA or cells that the UE enters |
| >Timestamp |  | A time stamp when the AMF detects the UE enters this location |
| Type Allocation code (TAC) | AMF | To indicate the terminal model and vendor information of the UE. The UEs with the same TAC may have similar mobility behavior. The UE whose mobility behavior is unlike other UEs with the same TAC may be an abnormal one. |
| Frequent mobility re-registration | AMF | A UE (e.g. a stationary UE) may re-select between neighbour cells due to radio coverage fluctuations. This may lead to multiple re-registrations if the cells belong to different registration areas. The number of re-registrations N within a period M may be an indication for abnormal ping-pong behaviour, where N and M are operator's configurable parameters. |

-	Service data related to UE mobility provided by AFs is defined in the Table 6.7.2.2-2;

Table 6.7.2.2-2: Service Data from AF related to UE mobility

| Information | Description |
| --- | --- |
| UE ID | Could be GPSI or external UE ID |
| Application ID | Identifying the application providing this information |
| UE trajectory (1..max) | Timestamped UE positions |
| >UE location | Geographical area that the UE enters |
| >Timestamp | A time stamp when UE enters this area |

Depending on the requested level of accuracy, data collection may be provided on samples (e.g. spatial subsets of UEs or UE group, temporal subsets of UE location information).

#### 6.7.2.3	Output Analytics

The NWDAF supporting data analytics on UE mobility shall be able to provide UE mobility analytics to consumer NFs or AFs. The analytics results provided by the NWDAF could be UE mobility statistics as defined in table 6.7.2.3-1, UE mobility predictions as defined in Table 6.7.2.3-2:

Table 6.7.2.3-1: UE mobility statistics

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, SUPI. |
| Time slot entry (1..max) | List of time slots during the Analytics target period |
| > Time slot start | Time slot start within the Analytics target period |
| > Duration | Duration of the time slot (average and variance) |
| > UE location (1..max) | Observed location statistics |
| >> UE location | TA or cells which the UE stays |
| >> Ratio | Percentage of UEs in the group (in case of an UE group) |

Table 6.7.2.3-2: UE mobility predictions

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies an UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, or SUPI. |
| Time slot entry (1..max) | List of predicted time slots |
| >Time slot start | Time slot start time within the Analytics target period |
| > Duration | Duration of the time slot |
| > UE location (1..max) | Predicted location prediction during the Analytics target period |
| >> UE location | TA or cells where the UE or UE group may move into |
| >> Confidence | Confidence of this prediction |
| >> Ratio | Percentage of UEs in the group (in case of an UE group) |

The results for UE groups address the group globally. The ratio is the proportion of UEs in the group at a given location at a given time.

The number of time slots and UE locations is limited by the maximum number of results provided as input parameter.

The time slots shall be provided by order of time, possibly overlapping. The locations shall be provided by decreasing value of ratio for a given time slot. The sum of all ratios on a given time slot must be equal or less than 100%. Depending on the list size limitation, the least probable locations on a given Analytics target period may not be provided.

#### 6.7.2.4	Procedures

The NWDAF can provide UE mobility related analytics, in the form of statistics or predictions or both, to another NF. If the NF is an AF, and when the AF is untrusted, the AF will request analytics via the NEF, and the NEF will then convey the request to NWDAF.

Figure 6.7.2.4-1: UE mobility analytics provided to an NF

1.	The NF sends a request to the NWDAF for analytics on a specific UE or a group of UEs, using either the Nnwdaf_AnalyticsInfo or Nnwdaf_AnalyticsSubscription service. The NF can request statistics or predictions or both. The type of analytics is set to UE mobility information. The NF provides the UE id or Internal Group ID in the Target of Analytics Reporting.

2.	If the request is authorized, and in order to provide the requested analytics, the NWDAF may subscribe to events with all the serving AMFs for notification of location changes. This step may be skipped when e.g. the NWDAF already has the requested analytics available.

NOTE:	The NWDAF determines the AMF serving the UE or the group of UEs as described in clause 6.2.2.1.

3.	The NWDAF derives requested analytics.

4.	The NWDAF provide requested UE mobility analytics to the NF, using either the Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1. The details for UE mobility analytics provided by NWDAF are defined in clause 6.7.2.3.

5-6. If at step 1, the NF has subscribed to receive notifications for UE mobility analytics, after receiving event notification from the AMFs subscribed by NWDAF in step 2, the NWDAF may generate new analytics and provide them to the NF.

### 6.7.3	UE Communication Analytics

#### 6.7.3.1	General

In order to support some optimized operations, e.g. customized mobility management, traffic routing handling, or QoS improvement, in 5GS, an NWDAF may perform data analytics on UE communication pattern and user plane traffic, and provide the analytics results (i.e. UE communication statistics or prediction) to NFs in the 5GC.

An NWDAF supporting UE Communication Analytics collects per-application communication description from AFs. If consumer NF provides an Application ID, the NWDAF only considers the data from AF, SMF and UPF that corresponds to this application ID.

The consumer of these analytics may indicate in the request:

-	The Target of Analytics Reporting which is a single UE or a group of UEs.

-	Analytics Filter Information including S-NSSAI, DNN.

-	An Analytics target period indicates the time period over which the statistics or predictions are requested.

-	Preferred level of accuracy of the analytics (low/high).

#### 6.7.3.2	Input Data

The NWDAF supporting data analytics on UE communication shall be able to collect communication information for the UE from 5GC. The detailed information collected by the NWDAF includes service data related to UE communication as defined in the Table 6.7.3.2-1.

Table 6.7.3.2-1: Service Data from 5GC related to UE communication

| Information | Source | Description |
| --- | --- | --- |
| UE ID | SMF, AF | SUPI in the case of SMF, GPSI or external UE ID in the case of AF |
| Group ID | SMF, AF | To identify UE group if available Internal Group ID in the case of SMF, External Group ID in the case of AF |
| S-NSSAI | SMF | Information to identify a Network Slice |
| DNN | SMF | Data Network Name where PDU connectivity service is provided |
| Application ID | SMF, AF | Identifying the application providing this information |
| Communication pattern parameters | AF | Same as communication pattern specified in TS 23.502 [3] |
| UE communication (1..max) | UPF, AF | Communication description per application |
| >Communication start |  | The time stamp that this communication starts |
| >Communication stop |  | The time stamp that this communication stops |
| >UL data rate |  | UL data rate of this communication |
| >DL data rate |  | DL data rate of this communication |
| >Traffic volume |  | Traffic volume of this communication |
| Type Allocation code (TAC) | AMF | To indicate the terminal model and vendor information of the UE. The UEs with the same TAC may have similar communication behavior. The UE whose communication behavior is unlike other UEs with the same TAC may be an abnormal one. |

NOTE:	How NWDAF collects UE communication related data is not defined in this Release of the specification.

Depending on the requested level of accuracy, data collection may be provided on samples (e.g. spatial subsets of UEs or UE group, temporal subsets of UE communication information).

#### 6.7.3.3	Output Analytics

The NWDAF supporting UE Communication Analytics provides the analytics results to consumer NFs. The analytics results provided by the NWDAF include the UE communication statistics as defined in Table 6.7.3.3-1 or predictions as defined in Table 6.7.3.3-2.

Table 6.7.3.3-1: UE Communication Statistics

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies an UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7 or SUPI. |
| UE communications (1..max) | List of communication time slots. |
| > Periodic communication indicator | Identifies whether the UE communicates periodically or not. |
| > Periodic time | Interval Time of periodic communication (average and variance) if periodic. Example: every hour |
| > Start time | Start time observed (average and variance) |
| > Duration time | Duration interval time of communication (average and variance). |
| > Traffic characterization | S-NSSAI, DNN, ports, other useful information. |
| > Traffic volume | Volume UL/DL (average and variance). |
| > Ratio | Percentage of UEs in the group (in case of an UE group). |

Table 6.7.3.3-2: UE Communication Predictions

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies an UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7 or SUPI. |
| UE communications (1..max) | List of communication time slots. |
| > Periodic communication indicator | Identifies whether the UE communicates periodically or not. |
| > Periodic time | Interval Time of periodic communication (average and variance) if periodic. Example: every hour. |
| > Start time | Start time predicted (average and variance). |
| > Duration time | Duration interval time of communication. |
| > Traffic characterization | S-NSSAI, DNN, ports, other useful information. |
| > Traffic volume | Volume UL/DL (average and variance). |
| > Confidence | Confidence of the prediction. |
| > Ratio | Percentage of UEs in the group (in case of an UE group). |

The results for UE groups address the group globally. The ratio is the proportion of UEs in the group for a given communication at a given time and duration.

The number of UE communication entries (1..max) is limited by the maximum number of results provided as input parameter. The communications shall be provided by order of time, possibly overlapping.

Depending on the list size limitation, the least probable communications on a given Analytics target period may not be provided.

#### 6.7.3.4	Procedures

The NWDAF can provide UE communication related analytics, in the form of statistics or predictions or both, to a 5GC NF.

Figure 6.7.3.4-1: Procedure for UE communication analytics

1.	5GC NF to NWDAF: Nnwdaf_AnalyticsSubscription_Subscribe (Analytics ID=UE communication analytics, Target of Analytics Reporting=SUPI(s), Analytics Filter=(Application ID, Area of Interest)).

5GC NF sends a request to the NWDAF for analytics on a specific UE(s), using either Nnwdaf_AnalyticsInfo or Nnwdaf_AnalyticsSubscription_Subscribe service. The analytics type indicated by "Analytics ID" is set to "UE communication". The Target of Analytics Reporting is set to SUPI or a list of SUPI and Analytics Filter may include Application ID and Area of Interest.

2a-b.	NWDAF to AF (Optional): Naf_EventExposure_Subscribe (Event ID, external UE ID, Application ID, Area of Interest).

In order to provide the requested analytics, the NWDAF may subscribe per application communication information, which is identified by Application ID, from AFs for the UE. The Event ID indicates communication report for the UE which is requested by the 5GC NF in the step 1. The external UE ID is obtained by the NWDAF based on UE internal ID, i.e., SUPI. In the case of external AF, the NEF translates the requested Area of Interest into a list of geographic zone identifier(s) as described in clause 5.6.7.1 of TS 23.501 [2].

This step is skipped if the NWDAF already has the requested analytics available or has subscribed to the AF.

2c-d.	NWDAF to SMF: Nsmf_EventExposure_Subscribe (Event ID, SUPI, Application ID, Area of Interest).

In order to provide the requested analytics, the NWDAF subscribes to PDU Session information of the UE from SMFs.

2e-f.	NWDAF to AMF: Namf_EventExposure_Request (Event ID, SUPI).

In order to provide the requested analytics, the NWDAF retrieves Type Allocation code from AMF.

NOTE:	The NWDAF determines the SMF serving the UE as described in clause 6.2.2.1.

3.	The NWDAF derives requested analytics, in the form of UE communication statistics or predictions or both.

4.	NWDAF to 5GC NF: Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify.

The NWDAF provides requested UE communication analytics to the NF, using either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1.

5-7.	If the NF subscribed UE communication analytics at step 1, when the NWDAF generates new analytics, it provides the new generated analytics to the NF.

### 6.7.4	Expected UE behavioural parameters related network data analytics

#### 6.7.4.1	General

The clause 6.7.4 defines how a service consumer learns from the NWDAF the expected UE behaviour parameters as defined in clause 4.15.6.3, TS 23.502 [3] for a group of UEs or a specific UE.

The service consumer may be an NF (e.g. AMF, UDM, AF), or the OAM.

The consumer of these analytics shall indicate in the request:

-	Analytics ID set to "UE Mobility" or "UE Communication".

-	The Target of Analytics Reporting can be a UE or an Internal Group Identifier.

NOTE:	In the case of untrusted AF the Target of Analytics Reporting can be a GPSI or an External Group Identifier that is mapped in the 5GC to a SUPI or an Internal Group Identifier

-	An Analytics target period, which indicates the time period over which the statistics or predictions are requested.

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

The NWDAF shall notify the result of the analytics to the consumer as indicated in clause 6.7.4.3.

#### 6.7.4.2	Input Data

In order to produce "UE Mobility" analytics, the NWDAF collects UE mobility information as defined in clause 6.7.2.2.

In order to produce "UE Communication" analytics, the NWDAF collects UE communication information as defined in clause 6.7.3.2.

#### 6.7.4.3	Output Analytics

The analytics results for "UE Mobility" are specified in Table 6.7.2.3-1.

The analytics results for "UE Communication" are specified in Table 6.7.3.3-1.

#### 6.7.4.4	Procedures

##### 6.7.4.4.1	NWDAF-assisted expected UE behavioural analytics

Figure 6.7.4.4.1-1: NWDAF assisted expected UE behavioural analytics procedure

1.	5GC NF (e.g., AMF, SMF, AF and UDM) to NWDAF: Nnwdaf_AnalyticsInfo_Request (Analytics ID, Target of Analytics Reporting=SUPI) or Nnwdaf_AnalyticsSubscription_Subscribe (Analytics ID, Target of Analytics Reporting =SUPI).

The Analytics ID is set to "UE Mobility" or to "UE Communication"," and the consumer request analytics.

2a-b.	[Conditional] The NWDAF requests to AMF Namf_EventExposure_Subscribe (Event ID, SUPI), to request location report for the UE, if the Analytics ID is set to "UE Mobility", unless the information is available.

2c-d.	[Conditional] The NWDAF requests to SMF Nsmf_EventExposure_Subscribe (Event ID, SUPI), to request communication report for the UE, if the Analytics ID is set to "UE Communication", unless the information is available.

3.	The NWDAF derives requested analytics.

4.	NWDAF to 5GC NF: Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify.

The NWDAF provides requested Expected UE behaviour to the NF, using either Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1.

5-6.	If the NF subscribed to at step 1, when the NWDAF generates new analytics, it provides the new generated analytics to the NF.

### 6.7.5	Abnormal behaviour related network data analytics

#### 6.7.5.1	General

This clause defines how to identify a group of UEs or a specific UE with abnormal behaviour, e.g. being misused or hijacked, with the help of NWDAF.

NOTE 1:	The misused or hijacked UEs are UEs in which there are malicious applications running or UEs which have been stolen.

The consumer of this analytics could be a 5GC NF. The 5GC NF subscribes analytics on abnormal behaviour from a NWDAF based on the UE subscription, network configuration or application layer request.

The NWDAF performs data analytics on abnormal behaviour if there is a related subscription.

The consumer of this analytics shall indicate in the request:

-	Analytics ID set to "Abnormal behaviour";

-	The Target of Analytics Reporting can be one UE, any UE or an Internal Group Identifier;

-	Optionally an Application ID, DNN or S-NSSAI;

-	An Analytics target period indicates the time period over which the statistics or predictions are requested;

-	Analytics Filter Information: optional expected UE behaviour, expected analytics type or list of Exception IDs with associated thresholds, where the expected analytics type can be mobility related, communication related or both, Area of interest, maximum number of results;

NOTE 2:	The expected analytics type generally indicates whether mobility or communication related abnormal behaviour analytics or both are expected by the consumer, and the list of exception IDs indicates what specific analytics are expected by the consumer, the expected analytics type and the list of Exception IDs are not presented simultaneously.

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

When the NWDAF detects those UEs that deviate from the expected UE behaviour, e.g. unexpected UE location, abnormal traffic pattern, wrong destination address etc., the NWDAF shall notify the result of the analytics to the consumer as specified in clause 6.7.5.3.

#### 6.7.5.2	Input Data

The Exceptions information from AF is as specified in Table 6.7.5.2-1.

On request of the service consumer, the NWDAF shall collect and analyse UE behavioural information and/or expected UE behavioural parameters from the 5GC NFs (SMF, AMF, AF), or OAM.

The UE behavioural information collected from 5GC NFs is as specified in clauses 6.7.2.2 and 6.7.3.2.

The expected UE behavioural parameters provided to the NWDAF are defined in clause 4.15.6.3, TS 23.502 [3].

Table 6.7.5.2-1: Exceptions information from AF

| Information | Description |
| --- | --- |
| IP address 5-tuple | To identify a data flow of a UE via the AF (such as the Firewall or a Threat Intelligence Sharing platform) |
| Exceptions (1..max) (NOTE) |  |
| >Exception ID | Indicating the Exception ID (such as Unexpected long-live/large rate flows and Suspicion of DDoS attack as defined in Table 6.7.5.3-2) of the data flow. |
| >Exception Level | Measured level, compared to the threshold |
| >Exception trend | Measured trend (up/down/unknown/stable) |
| NOTE: The Exceptions information and the UE behavioural information as defined in clauses 6.7.2.2 and 6.7.3.2 could help NWDAF to train an Abnormal classifier, which could be used to classify a UE behaviour data into Normal behaviour or Exception. |  |

#### 6.7.5.3	Output Analytics

The NWDAF services as defined in the clauses 7.2 and 7.3 are invoked to notify consumer NFs. A new Analytics ID named "Abnormal behaviour" is defined.

Corresponding to the Analytics ID, the analytics result provided by the NWDAF is defined in Table 6.7.5.3-1 and Table 6.7.5.3-2. When the level of an exception trespasses above or below the threshold, the NWDAF shall notify the consumer of the exception with an appropriate exception ID if the exception has an identity within the list of exception IDs indicated by the consumer or matches the expected analytics type indicated by the consumer. The NWDAF shall provide the Exception Level and determine which of the other information elements to provide, depending on the observed exception.

Abnormal behaviour statistics information is defined in Table 6.7.5.3-1.

Table 6.7.5.3-1: Abnormal behaviour statistics

| Information | Description |
| --- | --- |
| Exceptions (1..max) | List of observed exceptions |
| > Exception ID | The risk detected by NWDAF |
| > Exception Level | Measured level, compared to the threshold |
| > Exception trend | Measured trend (up/down/unknown/stable) |
| > UE characteristics | Internal Group Identifier, TAC |
| > SUPI list (1..N) | SUPI(s) of the UE(s) affected with the Exception |
| > Ratio | Estimated percentage of UEs affected by the Exception within the Target of Analytics Reporting |
| > Amount | Estimated number of UEs affected by the Exception (applicable when the Target of Analytics Reporting = "any UE") |
| > Additional measurement | Specific information for each risk |

Abnormal behaviour predictions information is defined in Table 6.7.5.3-2.

Table 6.7.5.3-2: Abnormal behaviour predictions

| Information | Description |
| --- | --- |
| Exceptions (1..max) | List of predicted exceptions |
| > Exception ID | The risk detected by NWDAF |
| > Exception Level | Measured level, compared to the threshold |
| > Exception trend | Measured trend (up/down/unknown/stable) |
| > UE characteristics | Internal Group Identifier, TAC |
| > SUPI list (1..N) | SUPI(s) of the UE(s) affected with the Exception |
| > Ratio | Estimated percentage of UEs affected by the Exception within the Target of Analytics Reporting |
| > Amount | Estimated number of UEs affected by the Exception (applicable when the Target of Analytics Reporting = "any UE") |
| > Additional measurement | Specific information for each risk |
| > Confidence | Confidence of this prediction |

The UE characteristics may provide a set of features common to all UEs affected with the exception.

The length of the SUPI list shall be lower than the parameter "maximum number of results" (see clause 6.1.3).

If PCF subscribes notifications on "Abnormal behaviour", the NWDAF shall send the PCF notifications about the risk, which may trigger the PCF to update the AM/SM policies.

The NWDAF also sends the notification directly to the AMF or SMF, if the AMF or SMF subscribes the notification, so that the AMF or SMF may, based on operator local policies defined on a per S-NSSAI or per (DNN,S-NSSAI), take actions for risk solving. The following Table 6.7.5.3-2 gives examples of additional measurement, AM/SM policies and corresponding actions for solving each risk.

Table 6.7.5.3-3: Examples of policies and actions for risk solving

| Exception ID and description | Additional measurement | AM/SM policy | Actions of NFs |
| --- | --- | --- | --- |
| Unexpected UE location | Unexpected UE location (TA or cells which the UE stays) | Add the area of current UE location into mobility restriction | PCF may extend the Service Area Restrictions. AMF may extend the mobility restriction |
| Unexpected long-live/large rate flows | Unexpected flow template (IP address 5 tuple) | Decrease the MBR for the related QoS flow | SMF updates the QoS rule. PCF, if dynamic PCC applies for corresponding DNN, S-NSSAI, updates PCC Rules that triggers SMF updates the QoS rule. |
| Unexpected wakeup | Time of unexpected wake-up | Apply MM back-off timer to the UE | AMF applies MM back-off timer to the UE |
| Suspicion of DDoS attack | Victim's address (target IP address list) | Release the PDU session and Apply SM back-off timer | PCF may request SMF to release the PDU session. SMF may release the PDU session and applies SM back-off timer |
| Wrong destination address | Wrong destination address (target IP address list) | Update the packet filter of the related QoS flow to block the wrong SDF | PCF updates the packet filter in the PCC Rules that triggers the SMF to update the related QoS flow and configures the UPF |
| Ping-pong UE | Numbers, frequency, time and location, assumptions about the possible circumstances | NWDAF notifies the AMF or AF (Service Provider) | AMF may adjust the UE (e.g. a stationary UE) registration area. |
| Too frequent Service Access/Abnormal traffic volume | Volume, frequency, time, assumptions about the possible circumstances | NWDAF notifies AF (Service Provider) |  |
| Unexpected radio link failures | Numbers, frequency, time and location, assumptions about the possible circumstances | Not applicable | If the unexpected radio link failures are per UE location bases, the AMF may allow the use of CE (Coverage Enhancement) in the affected location. Also, the Operator may improve the coverage conditions in the affected location. If the unexpected radio link failures are per UE bases, then the AMF and/or the AF may allow the use of CE for the affected UE. |
| Ping-ponging across neighbouring cells | Numbers, frequency, time and location information, assumption about the possible circumstances of the ping-ponging | Not applicable | If the amount of ping-ponging across neighbouring cells is above the thresholds set by the service provider, the service provider may adjust and improve the antenna tilts of the neighbouring cells or the overlapping coverage conditions in the affected location. If the ping-ponging are per UE, then the AMF and/or the AF may allow the use of Coverage Enhancement for the affected UE. |

#### 6.7.5.4	Procedure

Figure 6.7.5.4-1: Procedure for NWDAF assisted misused or hijacked UEs identification

1a.	A consumer NF subscribes to NWDAF using Nnwdaf_AnalyticsSubscription_Subscribe (Analytics ID set to "Abnormal behaviour", Target of Analytics Reporting = Internal-Group-Identifier or SUPI).

A consumer NF may subscribe to abnormal behaviour notification from NWDAF for a group of UEs or a specific UE. The Analytics ID indicates the NWDAF to identify misused or hijacked UEs through abnormal behaviour analytic.

1b.	AF to NWDAF: Nnwdaf_AnalyticsSubscription_Subscribe (Analytics ID, Target of Analytics Reporting = External-group identifier or External UE ID).

For untrusted AFs, the AF sends the subscription via a NEF, where the AF invokes NEF service Nnef_AnalyticsExposure_Subscribe (Analytics ID, Target of Analytics Reporting = External-group-identifier or External UE ID).

An AF may also subscribe to abnormal behaviour notification from NWDAF for a group of UEs or a specific UE, where the subscription message may contain expected UE behaviour parameters identified on the application layer. If an External-Group-Identifier is provided by the AF, the NEF interrogates UDM to map the External-Group-Identifier to the Internal-Group-Identifier and obtain SUPI list corresponding to the Internal-Group-Identifier.

2.	NWDAF to AMF (Conditional): Namf_EventExposure_Subscribe (Event ID(s), Event Filter(s), Internal-Group-Identifier or SUPI).

The NWDAF sends subscription requests to the related AMF to collect UE behavioural information if it has not subscribed such data.

NOTE 1:	The NWDAF determines the AMF serving the UE as described in clause 6.2.2.1.

The AMF sends event reports to the NWDAF based on the report requirements contained in the subscription request received from the NWDAF.

If requested by NWDAF via Event Filter(s), the AMF checks whether the UE's behaviour matches its expected UE behavioural information. In this case, the AMF sends event reports to the NWDAF only when it detects that the UE's behaviour deviated from its expected UE behaviour.

3.	NWDAF to SMF (Conditional): Nsmf_EventExposure_Subscribe (Event ID(s), Event Filter(s), Internal-Group-Identifier or SUPI).

The NWDAF sends subscription requests to the related SMF if it has not subscribed such data.

NOTE 2:	The NWDAF determines the SMF serving the UE as described in clause 6.2.2.1.

The SMF sends event reports to the NWDAF based on the report requirements contained in the subscription request received from the NWDAF.

If requested by NWDAF via Event Filter(s), the SMF checks whether the UE's behaviour matches its expected UE behavioural information. In this case, the SMF sends event reports to the NWDAF only when it detects that the UE's behaviour deviated from its expected UE behaviour.

4.	The NWDAF performs data analytics for misused or hijacked UEs identification. Based on the analytics and operator's policies the NWDAF determines whether to send a notification to 5GC NFs or the AF.

5a.	NWDAF to consumer NF (AMF or PCF or SMF depending on the subscription) (Conditional): Nnwdaf_AnalyticsSubscription_Notify (Analytics ID, (Exception ID, Internal-Group-Identifier or SUPI, Exception level)).

If the NWDAF determines to send a notification to the subscribed 5GC NFs, the NWDAF invokes Nnwdaf_EventSubscription_Notify services. Based on the notification, the 5G NFs adopt configured actions to resolve/mitigate/avoid the risks as described in the Table 6.7.5.3-1.

5b.	NWDAF to AF (Conditional): Nnwdaf_AnalyticsSubscription_Notify (Analytics ID, (Exception ID, External UE ID)).

If the NWDAF determines to send a notification to the subscribed AF, the NWDAF needs to include external UE ID of the identified UE into the notification message.

NOTE 3:	Based on the notification, the AF can adopt corresponding actions, e.g. adjusting recommended TCP Window Size, adjusting recommended Service Start and End.

## 6.8	User Data Congestion Analytics

### 6.8.1	General

The NWDAF can provide user data congestion related analytics, by one-time reporting or continuous reporting, in the form of statistics or predictions or both, to another NF. User Data Congestion related analytics can relate to congestion experienced while transferring user data over the control plane or user plane or both. A request for user data congestion analytics relates to a specific area or to a specific user. If the consumer of these analytics provides a UE ID, the NWDAF determines the area where the UE is located. The NWDAF then collects measurements per cell and uses the measurements to determine user data congestion analytics.

The request for user data congestion related analytics indicates the location area information where congestion related analytics is desired or indicates a UE Identity that can be used by the NWDAF to determine the location area information where congestion related analytics is desired. When the consumer of user data congestion related analytics subscribes to user data congestion related analytics, it may indicate a threshold and the NWDAF will provide analytics to the consumer when the congestion level crosses the threshold. The consumer can indicate an S-NSSAI in the request when congestion analytics are needed on a per slice level.

The service consumer may be an NF (e.g. NEF, AF).

The consumer of these analytics may indicate in the request or subscription:

-	Analytics ID set to "User Data Congestion".

-	Target of Analytics Reporting containing either a SUPI, or "any UE".

-	Analytics Filter Information containing:

-	Optional maximum number of results;

-	Area of Interest (list of TA or Cells) which restricts the area in focus (mandatory if Target Of Event Reporting is set to "any UE", optional otherwise);

-	Optional S-NSSAI, in order to obtain congestion analytics only on a given slice.

-	Optional Reporting Threshold, which applies only for subscriptions and indicates conditions on the congestion level (Network Status Indication, see clause 6.8.3) to be reached in order to be notified by the NWDAF.

-	An Analytics target period indicates the time period over which the statistics or prediction are requested, either in the past or in the future.

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

The NWDAF notifies the result of the analytics to the consumer as indicated in clause 6.6.3.

### 6.8.2	Input data

The detailed information collected by the NWDAF is defined in Table 6.8.2-1.

NOTE:	Performance Measurements defined in TS 28.552 [8] represent resource utilisation but do not, by themselves, indicate the event of congestion or congestion levels. The NWDAF collects measurements from the OAM and how the NWDAF derives Network Status Indication (NSI) is not specified.

Table 6.8.2-1: Service Data from the NF related to User Data Congestion Analytics

| Information | Source | Description |
| --- | --- | --- |
| UE Location | AMF | UE location information that NWDAF can use to derive the Area of Interest. |
| AMF ID | UDM | The AMF ID to be used for obtaining the UE's location. |
| Measurements | OAM | Performance Measurements that will be used by the NWDAF to determine congestion levels. Performance Measurements are related to information transfer over the user plane and/or the control plane (e.g. UE Throughput, DRB Setup Management, RRC Connection Number, PDU Session Management, and Radio Resource Utilization as defined in TS 28.552 [8]). The NWDAF may obtain measurements by invoking management services that are defined in TS 28.532 [6] and TS 28.550 [7]. |

Additionally, NWDAF may use statistics or predictions on service experience as specified in clause 6.4.3 as an input, e.g. for service experience in a given area or service experience for some specific applications such as high bandwidth applications.

### 6.8.3	Output analytics

The NWDAF outputs the user data congestion analytics for transfer over the user plane, for transfer over the control plane, or for both. The output may consist of statistics, predictions, or both. The detailed information provided by the NWDAF is defined in Table 6.8.3-1 for statistics and in Table 6.8.3-2 for predictions.

Table 6.8.3-1: User Data Congestion  statistics

| Information | Description |
| --- | --- |
| Area of Interest | A list of TAIs or Cell IDs |
| Analytics |  |
| >Type | User Plane or Control Plane |
| >Applicable Time Window | The time period that the analytics applies to |
| >Network Status Indication | Congestion Level |

Table 6.8.3-2: User Data Congestion predictions

| Information | Description |
| --- | --- |
| Area of Interest | A list of TAIs or Cell IDs |
| Analytics |  |
| >Type | User Plane or Control Plane |
| >Applicable Time Window | The time period that the analytics applies to |
| >Network Status Indication | Congestion Level |
| > Confidence information | Confidence of this prediction |

### 6.8.4	Procedures

#### 6.8.4.1	Procedure for one-time or continuous reporting of analytics for user data congestion in a geographic area

The procedure as depicted in Figure 6.8.4.1-1 is used by an NF to retrieve congestion analytics for a specific geographic area. The procedure can be used to request a one-time or continuous reporting of congestion analytics.

Figure 6.8.4.1-1: Procedure for one-time or continuous reporting of analytics for congestion in a geographic area

For one-time reporting:

1.	The NF sends Nnwdaf_AnalyticsInfo_Request to NWDAF, indicating request for analytics for congestion in a specific location. The NF can request statistics or predictions or both. The type of analytics is set to user data congestion analytics for transfer over user plane, control plane, or both, Analytics Filter is set to a location (e.g. ECGI, TA).

2-3. If the request is authorized, and in order to provide the requested analytics, the NWDAF may request the measurement information for the requested location from OAM services following the data collection from OAM procedure as captured in 6.2.3.2. If the NWDAF already has information about the requested location, these steps are omitted. The NWDAF may obtain measurements by invoking management services that are defined in TS 28.532 [6] and TS 28.550 [7].

4.	The NWDAF derives requested analytics.

5.	The NWDAF provides the analytics for congestion to the NF.

For continuous reporting:

6.	The NF sends Nnwdaf_AnalyticsSubscription_Subscribe Request to the NWDAF, indicating request for analytics for congestion in a specific location (e.g. ECGI, TA), possibly with thresholds. The NF can request statistics or predictions or both. The type of analytics is set to user data congestion analytics for transfer over user plane, control plane, or both.

7-8. The NWDAF subscribes to OAM services following the data collection from OAM procedure as captured in 6.2.3.2 to get measurement information for the requested location, possibly providing measurement thresholds. The NWDAF may obtain measurements by invoking management services that are defined in TS 28.532 [6] and TS 28.550 [7].

9.	The NWDAF derives requested analytics.

10.	The NWDAF provides the analytics for congestion to the NF.

11.	A change of user data congestion status corresponding to crossing a threshold set by the NWDAF is detected by OAM and notified to NWDAF.

12.	The NWDAF derives new analytics.

13.	The NWDAF provides a notification for analytics for the user data congestion to the NF.

#### 6.8.4.2	Procedure for one-time or continuous reporting of analytics for user data congestion for a specific UE

The procedure as depicted in Figure 6.8.4.2-1 is used by an NF to retrieve user data congestion analytics for a specific UE. The procedure can be used to request a one-time or continuous reporting of user data congestion analytics.

Figure 6.8.4.2-1: Procedure for one-time or continuous reporting of analytics for congestion for a specific UE

For one-time reporting:

1.	The NF sends Nnwdaf_AnalyticsInfo_Request to NWDAF, requesting for analytics for user data congestion for a specific UE id. The NF can request statistics or predictions or both. The type of analytics is set to user data congestion analytics for transfer over user plane, control plane, or both, the Target of Analytics Reporting is set to UE id.

2-5. The NWDAF may already know the UE location. If not, the NWDAF checks the UE location by first retrieving the AMF serving the UE (steps 2-3) and then by interrogating the AMF about the UE location.

6-7. The NWDAF requests measurement information for the UE location from OAM services (as captured in 6.2.3.2). These steps are omitted if the NWDAF already has the information. The NWDAF may obtain measurements by invoking management services that are defined in TS 28.532 [6] and TS 28.550 [7].

8.	The NWDAF derives requested analytics.

9.	The NWDAF provides the analytics for congestion to the NF.

For continuous reporting:

10.	The NF sends Nnwdaf_AnalyticsSubscription_Subscribe Request to the NWDAF. The NF can request for statistics or for predictions or for both. The type of analytics is set to user data congestion analytics for transfer over user plane, control plane, or both.

11.	The NWDAF determines the UE location, either via internal information or by applying the same steps as steps 2 to 5. The NWDAF then determines an area of interest.

12-13. The NWDAF subscribes to OAM services (as captured in 6.2.3.2) to get the measurement information for the UE location, possibly providing measurement thresholds. The NWDAF may obtain measurements by invoking management services that are defined in TS 28.532 [6] and TS 28.550 [7].

14.	The NWDAF derives requested analytics.

15.	The NWDAF provides the analytics for user data congestion status information to the NF.

16-17. The NWDAF subscribes to UE mobility event notification in order to be informed when the UE moves out of the area of interest (in order to define a new area of interest and request new information to OAM if the UE moves to a different area).

18.	A change of user data congestion status corresponding to crossing a threshold set by the NWDAF is detected by OAM and notified to NWDAF.

19.	The NWDAF derives new analytics.

20.	The NWDAF provides a notification for analytics for the user data congestion status information to the NF.

## 6.9	QoS Sustainability Analytics

### 6.9.1	General

The consumer of QoS Sustainability analytics may request the NWDAF analytics information regarding the QoS change statistics for an Analytics target period in the past in a certain area or the likelihood of a QoS change for an an Analytics target period in the future in a certain area. The consumer can request either to subscribe to notifications (i.e. a Subscribe-Notify model) or to a single notification (i.e., a Request-Response model). The request includes the following parameters:

-	Analytics ID = "QoS Sustainability";

-	Target of Analytics Reporting: "any UE";

-	Analytics Filter Information:

-	Optional maximum number of results;

-	QoS requirements:

-	5QI (standardized or pre-configured), and applicable additional QoS parameters and the corresponding values (conditional, i.e. it is needed for GBR 5QIs to know the GFBR); or

-	the QoS Characteristics attributes PDB, PER and their values;

-	Location information: an area or a path of interest. The location information could reflect a list of waypoints;

NOTE:	In this Release, the consumer of the "QoS Sustainability" Analytics ID will provide location information in the area of interest format (TAIs or Cell IDs) which is understandable by NWDAF.

-	Analytics target period: relative time interval, either in the past or in the future, that indicates the time period for which the QoS Sustainability analytics is requested;

-	S-NSSAI (optional);

-	Reporting Threshold(s), which apply only for subscriptions and indicate conditions on the level to be reached for the reporting of the analytics, i.e. to discretize the output analytics and to trigger the notification. The level(s) relate to the QoS KPIs, i.e. the RAN UE Throughput, the QoS flow Retainability, etc., for the relevant 5QI(s) defined in TS 28.554 [10]:

-	for a 5QI of GBR resource type, the Reporting Threshold(s) refer to the QoS flow Retainability KPI;

-	for a 5QI of non-GBR resource type, the Reporting Threshold(s) refer to the RAN UE Throughput KPI;

-	In a subscription, the Notification Correlation Id and the Notification Target Address.

The NWDAF collects the corresponding statistics information on the QoS KPI for the relevant 5QI of interests from the OAM, i.e. the QoS flow Retainability or the RAN UE Throughput as defined in TS 28.554 [10].

If the Analytics target period refers to the past:

-	The NWDAF verifies whether the triggering conditions for the notification of QoS change statistics are met and if so, generates for the consumer one or more notifications.

-	The analytics feedback contains the information on the location and the time for the QoS change statistics and the Reporting Threshold(s) that were crossed.

If the Analytics target period is in the future:

-	The NWDAF detects the need for notification about a potential QoS change based on comparing the expected values for the KPI of the target 5QI against the Reporting Threshold(s) provided by the consumer in any cell in the requested area for the requested Analytics target period. The expected KPI values are derived from the statistics for the 5QI obtained from OAM. OAM information may also include planned or unplanned outages detection and other information that is not in scope for 3GPP to discuss in detail.

-	The analytics feedback contains the information on the location and the time when a potential QoS change may occur and what Reporting Threshold(s) may be crossed.

### 6.9.2	Input data

Table 6.9.2-1: Data collection for "QoS Sustainability" analytics

| Information | Source | Description |
| --- | --- | --- |
| RAN UE Throughput | OAM TS 28.554 [10] | Average UE bitrate in the cell (Payload data volume on RLC level per elapsed time unit on the air interface, for transfers restricted by the air interface), per timeslot, per cell, per 5QI and per S-NSSAI. |
| QoS flow Retainability | OAM TS 28.554 [10] | Number of abnormally released QoS flows during the time the QoS Flows were used per timeslot, per cell, per 5QI and per S-NSSAI. |

NOTE:	The timeslot is the time interval split according to the time unit of the OAM statistics defined by operator.

### 6.9.3	Output analytics

The NWDAF outputs the QoS Sustainability analytics. Depending on the Analytics target period, the output consists of statistics or predictions. The detailed information provided by the NWDAF is defined in Table 6.9.3-1 for statistics and Table 6.9.3-2 for predictions.

Table 6.9.3-1: "QoS Sustainability" statistics

| Information | Description |
| --- | --- |
| Analytics |  |
| >Applicable Area | A list of TAIs or Cell IDs within the Location information that the analytics applies to. |
| >Applicable Time Period | The time period within the Analytics target period that the analytics applies to. |
| >Crossed Reporting Threshold(s) | The Reporting Threshold(s) that are met or exceeded by the statistics value or the expected value of the QoS KPI. |

Table 6.9.3-2: "QoS Sustainability" predictions

| Information | Description |
| --- | --- |
| Analytics |  |
| >Applicable Area | A list of TAIs or Cell IDs within the Location information that the analytics applies to. |
| >Applicable Time Period | The time period within the Analytics target period that the analytics applies to. |
| >Crossed Reporting Threshold(s) | The Reporting Threshold(s) that are met or exceeded by the statistics value or the expected value of the QoS KPI. |
| >Confidence | Confidence of the analytics. |

NOTE 1:	The meaning of Confidence is based on the SLA, i.e. the consumer has to understand the meaning of the different values of Confidence.

NOTE 2:	The Analytics can contain multiple sets of the above information in case the location information reflected a list of waypoints.

### 6.9.4	Procedures

Figure 6.9.4-1 depicts a procedure for "QoS Sustainability" analytics provided by NWDAF.

Figure 6.9.4-1: "QoS Sustainability" analytics provided by NWDAF

1.	The consumer requests or subscribes to analytics information on "QoS Sustainability" provided by NWDAF. The parameters included in the request are described in clause 6.9.1.

The consumer may include multiple sets of parameters in order to provide different combinations of "Location information" and "Analytics target period " when requesting QoS Sustainability analytics for monitoring for a path of interest.

2.	The NWDAF collects the data specified in clause 6.9.2 from the OAM.

3.	The NWDAF verifies whether the triggering conditions are met and derives the requested analytics. The NWDAF can detect the need for notification based on comparing the requested analytics of the target 5QI against the Reporting Threshold(s) provided by consumer in any cell over the requested Analytics target period.

4.	The NWDAF provides response or notification on "QoS Sustainability" to the consumer.

# 7	Nnwdaf Services Description

## 7.1	General

The following table illustrates the NWDAF Services.

Table 7.1-1: NF services provided by NWDAF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnwdaf_AnalyticsSubscription | Subscribe | Subscribe / Notify | PCF, NSSF, AMF, SMF, NEF, AF |
|  | Unsubscribe |  | PCF, NSSF, AMF, SMF, NEF, AF |
|  | Notify |  | PCF, NSSF, AMF, SMF, NEF, AF |
| Nnwdaf_AnalyticsInfo | Request | Request / Response | PCF, NSSF, AMF, SMF, NEF, AF |

The following table shows the analytics information provided by NWDAF service:

Table 7.1-2: Analytics information provided by NWDAF

| Analytics Information | Request Description | Response Description |
| --- | --- | --- |
| Slice Load level information | Analytics ID: load level information | Requested Analytics data, including load level information of Network Slice instance(s). |
| Observed Service experience information | Analytics ID: Service Experience | Observed Service experience may be provided, individually per UE or clustered per UE subsets in the case of group of UEs, or globally, averaged per Application or averaged across a set of Applications on a Network Slice. |
| NF Load information | Analytics ID: NF load information | Load statistics or predictions information for specific NF(s). |
| Network Performance information | Analytics ID: Network Performance | Statistics or predictions on the load in an Area of Interest; in addition, statistics or predictions on the number of UEs that are located in that Area of Interest. |
| UE mobility information | Analytics ID: UE Mobility | Statistics or predictions on UE mobility. |
| UE Communication information | Analytics ID: UE Communication | Statistics or predictions on UE communication. |
| Expected UE behavioural parameters | Analytics ID: UE Mobility and/or UE Communication | Analytics on UE Mobility and/or UE Communication. |
| UE Abnormal behaviour information | Analytics ID: Abnormal behaviour | The exception with an appropriate exception ID. |
| User Data Congestion information | Analytics ID: User Data Congestion | Statistics or predictions on the user data congestion for transfer over the user plane, for transfer over the control plane, or for both. |
| QoS Sustainability | Analytics ID: QoS Sustainability | For statistics, the information on the location and the time for the QoS change and the threshold(s) that were crossed; or, for predictions, the information on the location and the time when a potential QoS change may occur and what threshold(s) may be crossed. |

## 7.2	Nnwdaf_AnalyticsSubscription Service

### 7.2.1	General

Service Description: This service enables the consumer to subscribe/unsubscribe for network data analytics.

When the subscription is accepted by the NWDAF, the consumer NF receives from the NWDAF an identifier (Subscription Correlation ID) allowing to further manage (modify, delete) this subscription. The modification of Analytics subscription can be enforced by NWDAF based on operator policy and configuration.

### 7.2.2	Nnwdaf_AnalyticsSubscription_Subscribe service operation

Service operation name: Nnwdaf_AnalyticsSubscription_Subscribe.

Description: Subscribes to NWDAF analytics with specific parameters.

Inputs, Required: (Set of) Analytics ID(s) defined in Table 7.1-2, Target of Analytics Reporting, Notification Target Address (+ Notification Correlation ID), Analytics Reporting Parameters, Analytics target period.

Inputs, Optional: Analytics Filter Information, Subscription Correlation ID (in case of modification of the analytics subscription), preferred level of accuracy of the analytics.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription).

Outputs, Optional: None.

### 7.2.3	Nnwdaf_AnalyticsSubscription_Unsubscribe service operation

Service operation name: Nnwdaf_AnalyticsSubscription_Unsubscribe.

Description: unsubscribe to NWDAF analytic.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

### 7.2.4	Nnwdaf_AnalyticsSubscription_Notify service operation

Service operation name: Nnwdaf_AnalyticsSubscription_Notify.

Description: NWDAF notifies the consumer instance of the analytics that has subscribed to the specific NWDAF service.

Inputs, Required: Set of the tuple (Analytics ID, Analytics specific parameters), Notification Target Address (+ Notification Correlation ID), Notification Correlation Information.

Inputs, Optional: Timestamp for the analytics information, probability assertion.

NOTE:	Some NWDAF output analytics already include confidence of predictions, which provides the same information as probability assertion.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

## 7.3	Nnwdaf_AnalyticsInfo service

### 7.3.1	General

Service description: this service enables the consumer to request and get from NWDAF network data analytics.

### 7.3.2	Nnwdaf_AnalyticsInfo_Request service operation

Service operation name: Nnwdaf_AnalyticsInfo_Request.

Description: the consumer requests NWDAF operator specific analytics.

Inputs, Required: (Set of) Analytics ID(s) defined in Table 7.1-2, Target of Analytics Reporting, Analytics target period.

Inputs, Optional: Analytics Filter Information, preferred level of accuracy of the analytics, time when analytics information is needed.

Outputs, Required: Set of the tuple (Analytics ID, Analytics specific parameters).

Outputs, Optional: Timestamp for the analytics information, probability assertion.

NOTE:	Some NWDAF output analytics already include confidence of predictions, which provides the same information as probability assertion.

Annex A (informative):	Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-05 | SP#84 | SP-190456 | - | - | - | MCC Editorial update for presentation to TSG SA#84 for approval | 1.0.0 |
| 2019-06 | SP#84 | - | - | - | - | MCC editorial update for publication after approval at TSG SA#84 | 16.0.0 |
| 2019-09 | SP#85 | SP-190612 | 0001 | 3 | F | Clarifications to Observed Service experience related network data analytics | 16.1.0 |
| 2019-09 | SP#85 | SP-190612 | 0010 | 1 | F | Specification clean-up | 16.1.0 |
| 2019-09 | SP#85 | SP-190612 | 0012 | 3 | F | Miscellaneous corrections to TS 23.288 | 16.1.0 |
| 2019-09 | SP#85 | SP-190612 | 0014 | 1 | F | Clarification of NF and AF | 16.1.0 |
| 2019-09 | SP#85 | SP-190612 | 0015 | 3 | F | Update the Analytics information provided by NWDAF | 16.1.0 |
| 2019-09 | SP#85 | SP-190612 | 0017 | 2 | F | Closing open issue on NEF-AF interaction for data collection from AF | 16.1.0 |
| 2019-09 | SP#85 | SP-190612 | 0026 | 1 | F | Clarification of the correlation information | 16.1.0 |
| 2019-09 | SP#85 | SP-190612 | 0027 | 4 | F | Clarifications of the pre-check behaviours of the NF | 16.1.0 |
| 2019-09 | SP#85 | SP-190612 | 0029 | 3 | F | Corrections to slice load level analytics | 16.1.0 |
| 2019-09 | SP#85 | SP-190612 | 0034 | 3 | F | Clarifications on Potential QoS Change | 16.1.0 |
| 2019-09 | SP#85 | SP-190612 | 0036 | 1 | F | CR to properly separate UE identifiers from Analytics Filter | 16.1.0 |
| 2019-09 | SP#85 | SP-190612 | 0037 | 1 | F | CR for update of observed service experience | 16.1.0 |
| 2019-09 | SP#85 | SP-190612 | 0039 | 3 | F | Miscellaneous editorial corrections | 16.1.0 |
| 2019-09 | SP#85 | SP-190612 | 0040 | 3 | F | Optionality of data to be collected by NWDAF | 16.1.0 |
| 2019-09 | SP#85 | SP-190612 | 0042 | 1 | F | Clarification on Data Collection | 16.1.0 |
| 2019-09 | SP#85 | SP-190612 | 0045 | 1 | F | Probability assertion clarification on NWDAF services description | 16.1.0 |
| 2019-09 | SP#85 | SP-190612 | 0046 | 1 | F | Corrections for analytics exposure framework related parameters | 16.1.0 |
| 2019-09 | SP#85 | SP-190612 | 0052 | 1 | F | BSF and PCF selection for data collection | 16.1.0 |
| 2019-09 | SP#85 | SP-190612 | 0054 | - | F | Corrections to Nnwdaf_AnalyticsSubscription_Subscribe and Nnwdaf_AnalyticsInfo_Request service operations | 16.1.0 |
| 2019-12 | SP#86 | SP-191079 | 0002 | 6 | F | Clarifications to NF load data analytics | 16.2.0 |
| 2019-12 | SP#86 | SP-191079 | 0003 | 8 | F | Clarifications to Network Performance related network data analytics | 16.2.0 |
| 2019-12 | SP#86 | SP-191079 | 0004 | 3 | F | Clarifications to Abnormal behaviour analytics | 16.2.0 |
| 2019-12 | SP#86 | SP-191079 | 0009 | 4 | F | Clarifications to UE mobility and Abnormal behaviour analytics | 16.2.0 |
| 2019-12 | SP#86 | SP-191079 | 0043 | 2 | F | Remove UE related analytics for any UE | 16.2.0 |
| 2019-12 | SP#86 | SP-191079 | 0044 | 6 | F | Clarifications to UE communication and mobility analytics output | 16.2.0 |
| 2019-12 | SP#86 | SP-191079 | 0047 | 3 | F | Corrections for observed Service experience related network data analytics | 16.2.0 |
| 2019-12 | SP#86 | SP-191079 | 0055 | 01 | F | Terminology Alignment | 16.2.0 |
| 2019-12 | SP#86 | SP-191079 | 0057 | 5 | F | Editor's Notes cleanup | 16.2.0 |
| 2019-12 | SP#86 | SP-191079 | 0062 | - | F | Corrections to User Data Congestion Analytics | 16.2.0 |
| 2019-12 | SP#86 | SP-191079 | 0063 | - | F | Correction for data collection from OAM | 16.2.0 |
| 2019-12 | SP#86 | SP-191079 | 0064 | 7 | F | Corrections to general and framework parts of analytics | 16.2.0 |
| 2019-12 | SP#86 | SP-191079 | 0065 | - | F | Corrections to data collection from NFs | 16.2.0 |
| 2019-12 | SP#86 | SP-191079 | 0066 | 6 | F | Miscellaneous corrections/updates to TS 23.288 | 16.2.0 |
| 2019-12 | SP#86 | SP-191079 | 0068 | 4 | F | Clarification of the data collection of the OSE | 16.2.0 |
| 2019-12 | SP#86 | SP-191079 | 0071 | 3 | F | Update to UE related analytics | 16.2.0 |
| 2019-12 | SP#86 | SP-191079 | 0072 |  | F | Clarifications on Supporting Modification of Analytics Subscription | 16.2.0 |
| 2019-12 | SP#86 | SP-191079 | 0076 | 2 | F | Removing Editor's note on how to find a PCF instance serving a UE | 16.2.0 |
| 2019-12 | SP#86 | SP-191079 | 0078 | 2 | F | User Data Congestion - Removal of Editor's Notes and Description Alignments | 16.2.0 |
| 2019-12 | SP#86 | SP-191079 | 0081 | 3 | F | CR to update UE communication | 16.2.0 |
| 2019-12 | SP#86 | SP-191079 | 0084 | 3 | F | Correction to Analytics Filter for slice load level analytics | 16.2.0 |
| 2019-12 | SP#86 | SP-191079 | 0087 | 3 | F | Clarification on NWDAF-assisted expected UE behavioural analytics | 16.2.0 |
| 2019-12 | SP#86 | SP-191079 | 0088 |  | F | Update the correlation information for AMF data and RAN data | 16.2.0 |
| 2019-12 | SP#86 | SP-191079 | 0091 | 1 | F | Clarification of UE related analytics | 16.2.0 |
| 2019-12 | SP#86 | SP-191079 | 0092 |  | F | Clarification of QoS requirments parameter used for QoS Sustainabiltiy Analytics | 16.2.0 |
| 2019-12 | SP#86 | SP-191079 | 0093 | 4 | F | Alignments on Analytics Filter Information and clarifications on Reporting Thresholds | 16.2.0 |
| 2019-12 | SP#86 | SP-191079 | 0094 | 1 | F | Clarification for UPF related data collection | 16.2.0 |
| 2019-12 | SP#86 | SP-191120 | 0095 | 3 | F | Alignment of User Data Congestion Analytics | 16.2.0 |
| 2019-12 | SP#86 | SP-191079 | 0099 | 1 | F | NEF parameter mapping for outbound analytics | 16.2.0 |
| 2019-12 | SP#86 | SP-191079 | 0100 | 5 | F | Alignments on QoS Sustainability Analytics | 16.2.0 |