| 3GPP TS 23.435 V0.4.0 (2022-11) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Procedures for Network Slice Capability Exposure for Application Layer Enablement Service (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

Introduction	7

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	9

3.3	Abbreviations	9

4	Overview	9

4.1	Slice API configuration and translation	9

4.2	Application layer network slice lifecycle management	9

4.3	Network slice optimization based on AF policy	9

4.4	Discovery of management service exposure	9

4.5	Information collection from NSCE server(s)	10

4.x	<Functionality>	10

5	Business models and relationships for NSCALE	10

6	Architectural requirements	10

6.1	General Description	10

6.2	General requirements	10

6.3	Network slice Lifecycle management requirements	10

6.x	<Architectural requirements>	11

7	Application architecture for NSCALE	11

8	Identities and commonly used values	11

8.1	General	11

8.2	NSCE server ID	11

8.3	NSCE client ID	11

8.4	Slice coverage area	11

8.5	NSCE service area	11

9	Procedures and information flows	12

9.1	General	12

9.2	VAL server registration	12

9.2.1	General	12

9.2.2	Procedure	12

9.2.2.1	VAL server registration	12

9.2.2.2	VAL server de-registration	12

9.2.3	Information flows	12

9.2.3.1	General	12

9.2.3.2	VAL server registration request	12

9.2.3.3	VAL server registration response	13

9.2.3.4	Network slice capability de-registration request	13

9.2.3.5	Network slice capability de-registration response	13

9.2.4	APIs	14

9.2.4.1	General	14

9.2.4.2	Ensce_VALRegistration_Request operation	14

9.2.4.3	Ensce_VALRegistration_Deregister operation	14

9.3	Slice API configuration and translation	14

9.3.1	General	14

9.3.2	Procedures on slice API configuration	15

9.3.2.1	Initial Configuration	15

9.3.2.2	VAL server-initiated Configuration Update	16

9.3.3	Procedure on slice API translation	16

9.3.4	Information flows	17

9.3.4.1	General	17

9.3.4.2	VAL application requirement request	17

9.3.4.3	VAL application requirement response	18

9.3.4.4	Slice API information notify	18

9.3.4.5	VAL server-initiated trigger event	18

9.3.4.6	slice API invocation request	19

9.3.4.7	slice API invocation response	19

9.3.5	APIs	19

9.3.5.1	SS_VAL_Application_requirement API	20

9.3.5.1.1	General	20

9.3.5.1.2	VAL_Application_requirement	20

9.3.5.2	SS_Slice_API_notification API	20

9.3.5.1.1	General	20

9.3.5.1.2	Slice_API_information_notify	20

9.3.5.1	SS_Slice_API_invocation API	20

9.3.5.1.1	General	20

9.3.5.1.2	Slice_API_invocation	20

9.4	Application layer network slice lifecycle management	21

9.4.1	General	21

9.4.2	Procedure	21

9.4.3	Information flows	22

9.4.3.1	General	22

9.4.3.2	Application layer network slice lifecycle management subscribe request	22

9.4.3.3	Application layer network slice lifecycle management response	23

9.4.3.4	Application layer network slice lifecycle management notification	23

9.4.3.5	QoE metrics subscribe	24

9.4.3.6	QoE metrics response	24

9.4.3.7	QoE metrics notification	25

9.4.3.8	Network slice LCM recommendation request	25

9.4.3.9	Network slice LCM recommendation response	25

9.4.4	APIs	25

9.4.4.1	General	25

9.4.4.2	Ensce_AppLayerNSLCM_Subscribe operation	26

9.4.4.3	Ensce_AppLayerNSLCM_Notify operation	26

9.4.4.4	Eval_QoEMetrics_Subscribe operation	26

9.4.4.5	Eval_QoEMetrics_Notify operation	26

9.4.4.6	Eval_NSLCMRecommendation _Request operation	26

9.5	Network slice optimization based on AF policy	27

9.5.1	General	27

9.5.2	Procedure	27

9.5.2.1	 AF policy provisioning	27

9.5.2.2	 Network slice optimization based on AF policy	28

9.5.3	Information flows	29

9.5.3.1	General	29

9.5.3.2	AF policy provisioning request	29

9.5.3.3	AF policy provisioning response	30

9.5.3.4	Network slice optimization subscription request	30

9.5.3.5	Network slice optimization subscription response	31

9.5.3.6	Network slice optimization notify	31

9.5.4	APIs	31

9.5.4.1	General	31

9.5.4.2	Eval_AFPolicyProvisioning operation	31

9.5.4.6	Ensce_NSOptimization_Subscribe Request operation	32

9.5.4.3	Ensce_NSOptimization_Notify operation	32

9.6	Discovery of management service exposure	32

9.6.1	General	32

9.6.2	VAL-triggered MnS discovery procedure	32

9.6.3	OAM-triggered new/modified MnS discovery	33

9.6.4	Information flows	34

9.6.4.1	General	34

9.6.4.2	Management service discovery subscribe request	34

9.6.4.3	Management service discovery subscribe response	35

9.6.4.4	Management service discovery notify	35

9.6.5	APIs	35

9.6.5.1	SS_ Management_Service Discovery	36

9.6.5.1.1	General	36

9.6.5.1.2	Management_Service Discovery	36

9.7	Network slice related performance and analytics monitoring	36

9.7.1	General	36

9.7.2	Procedure	36

9.7.2.1	Network slice related performance and analytics monitoring request	36

9.7.2.2	Network slice related performance and analytics report subscription and report	38

9.7.3	Information flows	38

9.7.3.1	General	38

9.7.3.2	Network slice related performance and analytics monitoring request and response	38

9.7.3.3	Network slice related performance and analytics report subscription	39

9.7.3.4	Network slice related performance and analytics report	40

9.7.4	APIs	41

9.7.4.1	General	41

9.7.4.2	Ensce_PerfMonitoring API	41

9.7.4.3	Ensce_ PerfReportSubscription API	42

9.7.4.4	Ensce_ PerfReport API	42

9.8	Information collection from NSCE server(s)	42

9.8.1	General	42

9.8.2	Procedure	42

9.8.2.1	Information collection from NSCE server(s) subscribe request and response	42

9.8.2.2	Information collection from NSCE server(s) Notify	43

9.8.3	Information flows	43

9.8.3.1	General	43

9.8.3.2	Information collection from NSCE server(s) subscribe request	44

9.8.3.3	Information collection from NSCE server(s) subscribe response	44

9.8.3.4	Information collection from NSCE server(s) notify	44

9.8.4	APIs	45

9.8.4.1	General	45

9.8.4.2	SSnsce_InfoCollection_Subscribe operation	45

9.8.4.3	SSnsce_InfoCollection_Notify operation	45

9.x	<procedure name>	45

9.x.1	General	45

9.x.2	Procedure	45

9.x.3	Information flows	45

9.x.4	APIs (if applicable)	45

Annex A (informative): Deployment models	46

Annex B (informative): Change history	47

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

Network slice capability enablement (NSCE) is a service that enabling the network slice related capabilities towards 3rd party on the basis of R17 SEAL. The NSCE service providing additional functionality and expose slice capabilities based on SA5 MnS services and SA2 service (e.g. NEF APIs, NWDAF APIs, NSACF APIs). This technical specification provides procedures for enabling NSCE service over 3GPP networks while the architecture is defined in TS 23.434[2].

# 1	Scope

The present document specifies the procedures and information flows necessary for Network Slice Capability Exposure for Application Layer Enablement on the basis of R17 SEAL.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[3]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[4]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[5]	GSMA NG.116 - Generic Network Slice Template.

[6]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[7]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[8]		3GPP TS 28.531: "Management and orchestration; Provisioning".

[9]	3GPP TS 28.537: "Management and orchestration; Management capabilities".

[10]		3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

…

# 3	Definitions of terms, symbols and abbreviations

This clause and its three subclauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

# 4	Overview

Clauses under this clause shall document the functionalities of the Network Slice Capability Exposure for Application Layer Enablement.

## 4.1	Slice API configuration and translation

This functionality is provided to the vertical application specific layer and configures the exposure of APIs in a slice-tailored manner. It is assumed that the VAL server is not initially aware of the all the API exposure capabilities and information which will be needed for the given slice based on the SLA, and NSCE plays a vital role in configuring and translating the slice API based on the per slice requirements to service APIs. Slice API configuration and translation procedures are specified in clause 9.3.

## 4.2	Application layer network slice lifecycle management

Application layer network slice lifecycle management is provided to the VAL to better meet consumer's requirement without having to interact with 5GS frequently, based on network slice status collected from 5GS and QoE collected from application layer. Application layer network slice lifecycle management procedures are specified in clause 9.4.

## 4.3	Network slice optimization based on AF policy

Network slice optimization based on AF policy optimizes the network slice for the vertical applications by triggering the network slice modification exposed by EGMF defined in SA5. Network slice optimization based on AF policy procedures are specified in clause 9.5

## 4.4	Discovery of management service exposure

This network slice capability enablement feature supports the initial discovery of MnS for a given slice based on VAL server request, and the discovery of new/modified MnS with the required exposure (for example the permissions of the VAL server over the target MnS, e.g. read, write operations). This feature consists of two procedures, which are specified in clause 9.6:

- Discovery of management service capabilities and exposure level based on VAL server request

- Discovery and exposure of new or modified management service capabilities based on changes at OAM.

## 4.5	Information collection from NSCE server(s)

The network slice status collected by the NSCE server could be exposed to other NSCE server(s) if some agreement has been made. Information collection from NSCE server(s) procedures are specified in clause 9.8

## 4.x	<Functionality>

Add a copy of this clause for each functionality, adding a title and description.

# 5	Business models and relationships for NSCALE

Clauses under this clause shall document the business relationships for NSCALE.

# 6	Architectural requirements

Clauses under this clause shall document the architectural requirements for the architecture.

## 6.1	General Description

The following clauses specify the requirements for network slice capability enablement services for application layers.

## 6.2	General requirements

[AR-6.2-1] The NSCE architecture shall support the NSCE server to communicate with the VAL server with one or more applications.

[AR-6.2-2] The NSCE architecture shall support the NSCE server to communicate with one or more VAL servers.

[AR-6.2-3] The application enablement layer shall support the interactions with 3GPP network management system to consume network slice management service provided by MNO.

[AR-6.2-4] The APIs interactions between the vertical application server(s) and NSCE server(s) shall conform to CAPIF as specified in 3GPP TS 23.222 [3].

## 6.3	Network slice Lifecycle management requirements

[AR-6.3-1] The NSCE server shall enable lifecycle management capability exposure of network slice by authorized VAL server.

[AR-6.3-2] The NSCE architecture shall enable lifecycle management capability exposure of network slice communication service by authorized VAL server.

[AR-6.3-3] The NSCE server shall support the interactions with 3GPP network management system to consume network slice management service provided by MNO.

[AR-6.3-4] The NSCE server shall support the end-to-end network slice performance and analytics monitoring capability exposure to authorized VAL server.

[AR-6.3-5] The NSCE server shall support the network slice adaption capability exposure to authorized VAL server.

[AR-6.3-6] The NSCE server shall support the network slice adaption capability exposure to authorized NSCE client.

[AR-6.3-7] The NSCE server shall support the network slice configuration capability exposure to authorized VAL server.

[AR-6.3-8] The NSCE server shall support the network slice translation capability to enable the translation of slice API invocated by VAL server.

## 6.x	<Architectural requirements>

Add a copy of this clause for each architectural requirement, adding a title and description.

# 7	Application architecture for NSCALE

The application architecture are specified in clause 16 TS 23.434 [2].

# 8	Identities and commonly used values

## 8.1	General

The common identities for SEAL refer to TS 23.434[2]. The following clauses list the additional identities and commonly used values for Network Slice Capability Enablement.

## 8.2	NSCE server ID

The NSCE server ID uniquely identifies the Network Slice Capability enablement server.

## 8.3	NSCE client ID

The Network Slice Capability enablement client ID identifies a particular NSCE client.

## 8.4	Slice coverage area

The slice coverage area is the area where the network slice is available in the whole PLMN or in one or more Tracking Areas of the PLMN. The slice coverage area can be expressed as a Topological Service Area (e.g. a list of TA), a Geographical Service Area (e.g. geographical coordinates) or both.

## 8.5	NSCE service area

The NSCE service area is the area where the Network Slice Capability Enablement server owner provides its services. It is equal to the collection of coverage area of slices it can enable.

The NSCE service area can be expressed as a Topological Service Area (e.g. a list of TA), a Geographical Service Area (e.g. geographical coordinates) or both.

NOTE:	The NSCE server service area shall not smaller than the collection of slice(s) coverage area(s) the NSCE server can enable.

# 9	Procedures and information flows

Subclause of this clause shall describe the procedures for the application architecture for enabling Edge Application.

## 9.1	General

## 9.2	VAL server registration

### 9.2.1	General

The NSCE layer supports the exposure of network slice related capabilities to the VAL. To access network slice related enablement capabilities, e.g. monitoring of network slice performance, the VAL server should register on the NSCE layer.

### 9.2.2	Procedure

#### 9.2.2.1	VAL server registration

For registration of the VAL server to be a recognized user of the NSCE server, the VAL server triggers registration procedure which is defined as onboarding the API invoker in TS 23.222[3] clause 8.1.3. In such procedure, the NSCE server could play the role of CAPIF core function.

#### 9.2.2.2	VAL server de-registration

For de-registration of the VAL server, the VAL server triggers de-registration procedure which is defined as offboarding the API invoker in TS 23.222[3] clause 8.2.3.

### 9.2.3	Information flows

#### 9.2.3.1	General

The following information flows are specified for VAL server registration:

-	VAL server registration request and response; and

-	VAL server de-registration request and response.

#### 9.2.3.2	VAL server registration request

Table 9.2.3.2-1 describes information elements for the VAL server registration request from the VAL server to the NSCE server.

Table 9.2.3.2-1: VAL server registration request

| Information element | Status | Description |
| --- | --- | --- |
| Registration information | M | The information of the VAL server |
| >VAL server ID | M | The identifier of the VAL server |
| >VAL Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the VAL. |
| >Security credentials | M | Security credentials resulting from a successful authorization for the NSCE service. |
| >Proposed expiration time | O | Proposed expiration time for the registration |
| APIs for enrollment | O | Indicates interested NSCE services. |

#### 9.2.3.3	VAL server registration response

The information elements specified in the Table 9.2.3.3-1-1 are used for the VAL server registration response sent from the NSCE server to the VAL server.

Table 9.2.3.3-1: VAL server registration response

| Information element | Status | Description |
| --- | --- | --- |
| Registration status | M | The result of Registration request |
| >Successful response (NOTE) | O | Indicates that the VAL server registration request was successful. |
| > >Registration ID | M | Identifier of the registration. |
| > >Expiration time | O | Indicates the expiration time of the registration. To maintain an active registration status, a registration update is required before the expiration time. If the Expiration time IE is not included, it indicates that the registration never expires. |
| >>Registration information | M | Information allow the VAL server to be authenticated and to obtain authorization for service APIs |
| >>Service API information | O | NSCE service API name, service API type, communication type, description, Serving Area Information (optional), AEF location (optional), interface details (e.g. IP address, port number, URI), protocols, version numbers, and data format. |
| >Failure response (NOTE) | O | Indicates that the VAL server registration request failed. |
| > >Cause | O | Indicates the cause of VAL server registration request failure |
| NOTE: Only one of these IEs shall be present in the message. |  |  |

#### 9.2.3.4	Network slice capability de-registration request

Table 9.2.3.4-1 describes information elements for the network slice capability de-registration request from the VAL server to the NSCE server.

Table 9.2.3.4-1: Network slice capability de-registration request

| Information element | Status | Description |
| --- | --- | --- |
| Registration ID | M | Identifier of the registration. |
| Security credentials | M | Security credentials of the EAS |
| Reason | O | Indicate the reason of de-registration |

#### 9.2.3.5	Network slice capability de-registration response

The information elements specified in the Table 9.2.3.5-1-1 are used for the Network slice capability de-registration response sent from the NSCE server to the VAL server.

Table 9.2.3.5-1: Network slice capability de-registration response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (NOTE) | O | Indicates that the Network slice capability de-registration request was successful. |
| Failure response (NOTE) | O | Indicates that the Network slice capability de-registration request failed. |
| > Cause | O | Indicates the cause of Network slice capability de-registration request failure |
| NOTE: Only one of these IEs shall be present in the message. |  |  |

### 9.2.4	APIs

#### 9.2.4.1	General

Table 9.2.4.1-1 illustrates the API for VAL server registration.

Table 9.2.4.1-1: Ensce_VALRegistration API

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Ensce_VALRegistration | Request | Request/Response | VAL |
|  | Deregister |  |  |

#### 9.2.4.2	Ensce_VALRegistration_Request operation

API operation name: Ensce_VALRegistration_Request

Description: The consumer requests to register the VAL server on the NSCE server.

Inputs: See clause 9.2.3.2.

Outputs: See clause 9.2.3.3.

See clause 9.2.2.1 for details of usage of this operation.

#### 9.2.4.3	Ensce_VALRegistration_Deregister operation

API operation name: Ensce_VALRegistration_Deregister

Description: The consumer requests to deregister the VAL server from the NSCE server.

Inputs: See clause 9.2.3.4.

Outputs: See clause 9.2.3.5.

See clause 9.2.2.2 for details of usage of this operation.

## 9.3	Slice API configuration and translation

### 9.3.1	General

This functionality is a service related to the translation of the service API as invoked by the end applications to slice APIs based on the API configuration and application to slice mapping. Slice APIs can be defined as customized/tailored sets of service APIs (which can be either NEF northbound APIs or OAM provided APIs or enabler layer/SEAL provided APIs) and can be mapped to particular slice instances. The slice APIs can be a bundled or combined API comprising of different types of APIs, which will be used to expose the telco (5GS/SEAL)-provided services as needed by the applications of the slice customer. Each slice API may be configured per network slice instance.

### 9.3.2	Procedures on slice API configuration

In the Initial Configuration procedure, the VAL server initially provides an application requirement to enabler server including the service KPIs and the subscribed/preferred slices. Then, the slice enabler configures the mapping of the VAL application to a slice API which is a combination/bundling of northbound APIs (from both management and control plane). In particular, a slice API consists of telco-provided/platform dependent service APIs (e.g., NEF, OAM, SEAL, etc), and provides an abstraction/simplification on top of them. The VAL server-initiated Configuration Update  procedure covers the scenario where a trigger event occurs (e.g., QoS degradation, slice load) and the mapping configuration or the slice API configuration needed to be changed. In this scenario, the slice enabler updates the configuration of the API and provides a notification to the VAL server.

These two procedures for the initial configuration and the configuration update are covered in 9.3.2.1 and 9.3.2.2 respectively.

#### 9.3.2.1	Initial Configuration

Figure 9.3.2.1-1 illustrates the procedure of  the initial slice API configuration.

Pre-conditions:

1.	The VAL server has registered to receive NSCE services.

Figure 9.3.2.1-1: Initial Slice API configuration

1.	The VAL server sends a VAL application requirement request to the NSCE server.

2.	The NSCE server maps the VAL application requirement to a slice API which includes a list of APIs which is needed to be consumed as part of this service capability exposure.

The NSCE server may also store the mapping of the slice API to the service API list and per service API information (e.g. data encoding, transport technology, API protocol and versions)

3.	The NSCE server registers to consume the corresponding APIs from the 5GS (NEF and OAM) and SEAL service producers. The NSCE server registers to the following:

-	to consume NEF monitoring events as specified in 3GPP TS 29.522 clause 5 e.g.,  network monitoring, slice status, analytics exposure, etc

-	to consume PM services and KPI monitoring from OAM

-	to consume SEAL services based on 3GPP TS 23.434

4.	The NSCE server sends a VAL application requirement response to notify on the result of the request and indicate whether the configuration of slice API is possible or not.

5.	The NSCE server sends the slice API information notification to the VAL server.

#### 9.3.2.2	VAL server-initiated Configuration Update

Figure 9.3.2.2-1 illustrates the procedure of the slice API configuration updated based on a trigger event.

Figure 9.3.2.2-1: Slice API configuration update

1.	A trigger event is captured by the VAL server side (application server relocation to different EDN/DN, UE mobility to different EDN, application change of behaviour).

2.	The NSCE server processes the trigger event and checks the feasibility of such change and updates the mapping of service APIs to the slice APIs. One criterion for the update of the mapping is, if possible, to avoid changing the slice API configuration, which can be achieved by the re-mapping of the underlying service APIs

3.	The NSCE server updates the subscription/registration to the underlying 5GS and SEAL service producers, if an update on the service APIs (e.g., NEF APIs, SEAL APIs, OAM provided APIs) is needed.

4.	The NSCE server optionally sends a slice API information update notification to the VAL server.

### 9.3.3	Procedure on slice API translation

This procedure follows the 9.3.2 and aims to describe how the slice API invocation request is translated to service API invocations after the slice API configuration mapping. In this procedure, the NSCE server initially receives a slice API invocation request from the vertical application. Then, the NSCE server fetches the service APIs to be invoked based on the slice API configuration and performs invocation requests to the corresponding service API providers.

Figure 9.3.3-1 illustrates the procedure of the slice API translation based on the initial configuration.

Pre-conditions:

1.	The VAL server has registered to receive NSCE services.

2.	The slice API mapping to the VAL server has been performed based on 9.3.2.1 step 2 and the slice API information is provided to the VAL server based on 9.3.2.1 step 5.

Figure 9.3.3-1: Slice API translation

1.	The VAL server sends a slice API invocation request to the NSCE server

2.	The NSCE server checks that the user is authenticated and authorized to perform the slice API invocation and maps the requested slice API to a service APIs. If CAPIF is used, the NSCE server acts as AEF, and the authorization is obtained by CCF.

3.	The NSCE server generates a trigger for service API invocation requests to all the service APIs within the slice API.

4.	The NSCE server performs the corresponding service API invocation procedures based on CAPIF or via performing requests to the corresponding service API providers, which are mapped to the slice API.	If CAPIF is used, the requests are sent to the corresponding AEFs of the API provider's domain, and the authorization is obtained by CCF.

5.	The NSCE server sends a slice API invocation response to the VAL server, based on the result of the service API invocation response(s) of step 4.

### 9.3.4	Information flows

#### 9.3.4.1	General

The following information elements are specified for slice API translation and configuration specified in clause 9.3.2 and 9.3.3.

#### 9.3.4.2	VAL application requirement request

Table 9.3.4.2-1 describes information elements for the VAL application requirement request from the VAL server to the NSCE server.

This request provides the service requirements / KPIs, the capability exposure requirements and a preferred/subscribed slice identification (e.g., S-NSSAI or ENSI).

Table 9.3.4.2-1: VAL application requirement request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | The identifier of the VAL server |
| Service requirements | M | The VAL application requirements |
| > VAL server KPIs | O | KPIs including application QoS requirements (latency, error rates, throughput, jitter,..) |
| > Slice ID (NOTE) | M | S-NSSAI or ENSI for which the request applies |
| > NSI ID (NOTE) | O | If the request applies for a certain NSI, the identifier of the NSI shall be present |
| > Service Profile | O | If known by the VAL server, the Service Profile can be provided to the NSCE server. |
| Area of Interest | O | The geographical or service area for which the requirement request applies |
| Time validity | O | The time validity of the request |
| NOTE: One of this shall be present |  |  |

#### 9.3.4.3	VAL application requirement response

Table 9.3.4.3-1 describes information elements for the VAL application requirement response from the NSCE server to the VAL server.

Table 9.3.4.3-1: VAL application requirement response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of the application requirements request. |
| Successful response (NOTE) | O | Indicates that the VAL application requirement request was successful. |
| Failure response (NOTE) | O | Indicates that the VAL application requirement request failed. |
| >Cause | O | Indicates the cause of failure |
| NOTE : Only one of these IEs shall be present in the message. |  |  |

#### 9.3.4.4	Slice API information notify

Table 9.3.4.4-1 describes information elements for the Slice API information notification from the NSCE server to the VAL server. The Slice API information notification is used by NSCE server to send Slice API information to VAL server in the Initial Configuration and Configuration Update procedures.

Table 9.3.4.4-1: Slice API information notify

| Information element | Status | Description |
| --- | --- | --- |
| Slice API info | M | The information for the configured slice API |

#### 9.3.4.5	VAL server-initiated trigger event

Table 9.3.4.5-1 describes information elements for the Trigger Event from the VAL server to the NSCE server.

Table 9.3.4.5-1: VAL server-initiated trigger event

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | The identifier of the VAL server |
| Trigger Event Details | M | The trigger event details can be included, providing the cause for the need slice API configuration adaptation. Such trigger event can be: - the UE mobility to a different service area (outside the original area of interest), - application server migration to different edge/cloud platform, - service API unavailability, - VAL application QoS requirements change |

#### 9.3.4.6	slice API invocation request

Table 9.3.4.6-1 describes information elements for the slice API invocation request from the VAL server to the NSCE server.

Table 9.3.4.6-1: slice API invocation request

| Information element | Status | Description |
| --- | --- | --- |
| API invoker identity information | M | The information that determines the identity of the API invoker |
| Authorization information | O | The authorization information obtained before initiating the slice API invocation request |
| Slice API identification | M | The identification information of the slice API for which invocation is requested. The slice API identification is part of the specific slice API invocation request. |

#### 9.3.4.7	slice API invocation response

Table 9.3.4.7-1 describes information elements for the slice API invocation response from the NSCE server to the VAL server.

Table 9.3.4.7-1: slice API invocation response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of slice API invocation. |
| Successful response (NOTE) | O | Indicates that the slice API invocation request was successful. |
| Failure response (NOTE) | O | Indicates that the slice API invocation request failed. |
| >Cause | O | Indicates the cause of failure |
| NOTE : Only one of these IEs shall be present in the message. |  |  |

### 9.3.5	APIs

Table 9.3.5-1 illustrates the NSCALE APIs for the slice API configuration and translation feature.

Table 9.3.5-1: List of APIs for slice API configuration and translation feature

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_VAL_Application_requirement | VAL_Application_requirement | VAL server | Request /Response |
| SS_Slice_API_notification | Slice_API_information_notify | VAL server | Subscribe/Notify |
| SS_Slice_API_invocation | Slice_API_invocation | VAL server | Request /Response |

#### 9.3.5.1	SS_VAL_Application_requirement API

##### 9.3.5.1.1	General

API description: This API enables the VAL server to communicate with the NSCE server for providing the slice related application requirement over NSCE-S.

##### 9.3.5.1.2	VAL_Application_requirement

API operation name: VAL_Application_requirement

Description: Providing for VAL_Application_requirement to the NSCE server.

Known Consumers: VAL server.

Inputs: See subclause 9.3.4.2

Outputs: See subclause 9.3.4.3

#### 9.3.5.2	SS_Slice_API_notification API

##### 9.3.5.1.1	General

API description: This API enables the NSCE server to communicate with the VAL server for notifying the slice API information over NSCE-S.

##### 9.3.5.1.2	Slice_API_information_notify

API operation name: Slice_API_information_notify

Description: Notifying the slice API information

Known Consumers: VAL server.

Inputs: -

Outputs: See subclause 9.3.4.4

#### 9.3.5.1	SS_Slice_API_invocation API

##### 9.3.5.1.1	General

API description: This API enables the VAL server to communicate with the NSCE server for invoking the slice API over NSCE-S.

##### 9.3.5.1.2	Slice_API_invocation

API operation name: SS_Slice_API_invocation

Description: Requesting slice API invocation from NSCE server.

Known Consumers: VAL server.

Inputs: See subclause 9.3.4.6

Outputs: See subclause 9.3.4.7

## 9.4	Application layer network slice lifecycle management

### 9.4.1	General

When NSCE receives a request for application layer network slice lifecycle management (AppLayer_NS_LCM) from VAL server, the NSCE server performs the service operations including subscribing the event which may trigger the management system to do the network slice lifecycle management, making the network slice lifecycle management recommendation/decision, triggering the network slice lifecycle management operations, notifying the consumer about the network slice information.

### 9.4.2	Procedure

Figure 9.4.2-1 illustrates a procedure of application layer network slice lifecycle management based on network slice related data and QoE collected from application layer.

Pre-conditions:

1.	The NSCE server has authenticated and authorized to the capabilities to collect current network slice from 5GS.

2.	The NSCE server has authenticated and authorized to the capabilities trigger the network slice LCM operations.

3.	There is signed contract for LCM between the entities using VAL server and entities using NSCE.

Figure 9.4.2-1: Application layer network slice lifecycle management

1.	The VAL server sends the application layer network slice lifecycle management (AppLayer-NS-LCM) subscribe request to NSCE server, with network slice requirements from VAL server/consumer (e.g. delay, throughput, load, the maximum number of users supported, etc.). The request can indicate whether the notification is needed before performing the AppLayer-NS-LCM. The request can also indicate the trigger conditions, such as by providing the monitored parameters and the corresponding thresholds as described in clause 9.4.3.2.

2.	After receiving the request, the NSCE server checks that the user is authenticated and authorized to perform the corresponding AppLayer-NS-LCM operations, and filters the unauthorized requests, if any.

3.	According to network slice requirements and/or the trigger conditions, NSCE server triggers the provision of network slice status and QoE metrics. If the trigger conditions are not indicated in the subscription, the NSCE server can help to configure an appropriate trigger condition, such as report period or thresholds.

3a.	The network slice status could be collected through subscribing or requesting to 5GS. List of network slice status parameters to be collected are:

-	Network Slice load statistics information, and/or Network Slice load predictions information from NWDAF/NEF as defined in TS 23.288 Table 6.3.3A.

-	Performance metric in the performance data file from OAM as defined in clause 11.3 of TS 28.532 [7].

3b.	Also, the NSCE server could get the information of QoE metrics from the application layer domain by QoE metric subscribe as described in clause 9.4.3.5.

4.	Once authenticated and authorized, the NSCE server sends the AppLayer-NS-LCM response to the VAL server.

5.	The NSCE server may process and combine the collected network slice status and QoE metrics, if needed. The NSCE server may process and combine the parameters in the trigger conditions, if multiple trigger conditions were provided. Once the trigger condition or the combination of trigger conditions are met which indicating the network slice requirements from VAL server is not satisfied, the NSCE server determines whether and what network slice LCM operations should be taken based on requirements from VAL server/consumer and network slice status and QoE metrics, and makes the decision(s)/recommendation(s), such as modifyNsi/AllocateNsi/DeallocateNsi request as specified in TS 28.531 [8].

6.	Optionally, if it is indicated in the request to notify the VAL server/consumer before performing the AppLayer-NS-LCM, the NSCE sends the network slice LCM recommendation(s) with network slice information to VAL server, to see whether takes the recommendation(s) or not.

7.	Optionally, the VAL server sends the response to NSCE server.

8.	Based on decision made by VAL server or NSCE server, the NSCE server sends the network slice LCM request to the OAM.

9.	According to the corresponding operation(s) result, the NSCE server sends the response to the VAL server.

### 9.4.3	Information flows

#### 9.4.3.1	General

The following information flows are specified for AppLayer_NS_LCM:

-	AppLayer_NS_LCM subscribe, response and notification;

-	QoE metrics subscribe, response and notification;

-	Network slice LCM recommendation request and response;

#### 9.4.3.2	Application layer network slice lifecycle management subscribe request

Table 9.4.3.2-1 describes information elements for the application layer network slice lifecycle management subscribe request from the VAL server to the NSCE server.

Table 9.4.3.2-1: AppLayer-NS-LCM Subscribe

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Identifier | M | Unique identifier of the requestor (i.e. VAL server ID). |
| Security credentials | M | Security credentials resulting from a successful authorization for the NSCE service. |
| Notification Target Address | O | The Notification Target Address (e.g. URL) where the notifications destined for the requestor should be sent to. |
| Network slice requirements | M | The requirement of network requirements or updated requirements when the network slice was created. The GST defined by GSMA (see clause 2.2 in [5]) and the performance requirements defined in clause 7 TS 22.261 [6] are all considered as input for the network slice related requirements. |
| List of S-NSSAI(s) | M | Identifier of the interested network slice |
| AppLayer-NS-LCM notification indication | M | Indicates whether to notify the VAL server/consumer about the AppLayer-NS-LCM |
| >AppLayer-NS-LCM notification address (NOTE) | O | The address (e.g. URL) of the consumer that can receive the AppLayer-NS-LCM notification |
| Trigger condition | O | Indicates the monitored parameters and the corresponding thresholds which could trigger the AppLayer-NS-LCM. The supported trigger conditions are: - The Network Slice load exceeds the threshold. - The collected Network Slice performance exceeds the threshold; - The collected QoE exceeds the threshold. |
| Proposed expiration time | O | Proposed expiration time for the subscription |
| NOTE: When the AppLayer-NS-LCM recommendation notification is needed, AppLayer-NS-LCM notification indication should be provided. |  |  |

#### 9.4.3.3	Application layer network slice lifecycle management response

The information elements specified in the Table 9.4.3.3-1 is used for the application layer network slice lifecycle management response sent from the NSCE server to the VAL server.

Table 9.4.3.3-1: AppLayer-NS-LCM Response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (NOTE) | O | Indicates that the AppLayer-NS-LCM subscribe request was successful. |
| Failure response (NOTE) | O | Indicates that the AppLayer-NS-LCM subscribe request failed. |
| > Cause | O | Indicates the cause of AppLayer-NS-LCM request failure |
| NOTE: One of these IEs shall be present in the message. |  |  |

#### 9.4.3.4	Application layer network slice lifecycle management notification

The information elements specified in the Table 9.4.3.4-1 is used for the application layer network slice lifecycle management notification sent from the NSCE server to the VAL server.

Table 9.4.3.4-1: AppLayer-NS-LCM Notification

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (NOTE) | O | Indicates that the AppLayer-NS-LCM request was successful. |
| >network slice information | M | Network slice information (i.e. NEST) with network slice identifier(i.e. S-NSSAI) |
| Failure response (NOTE) | O | Indicates that the AppLayer-NS-LCM request failed. |
| > Cause | O | Indicates the cause of AppLayer-NS-LCM request failure |
| NOTE: One of these IEs shall be present in the message. |  |  |

#### 9.4.3.5	QoE metrics subscribe

Table 9.4.3.5-1 describes information elements for the QoE metrics subscribe from the NSCE server to the VAL server.

Table 9.4.3.5-1: QoE metrics Subscribe

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Identifier | M | Unique identifier of the requestor (i.e. NSCE server ID). |
| Security credentials | M | Security credentials resulting from a successful authorization. |
| Notification Target Address | O | The Notification Target Address (e.g. URL) where the notifications destined for the requestor should be sent to. |
| Subscription ID | M | Identifier of the subscription. |
| Event Filter | M | The associated filter on a network slice to be notified |
| >S-NSSAI(s) | M | Identifier of the interested network slice |
| >Application indicator | O | Indicator of the interested application (i.e. App ID) |
| >QoE type indicator | M | QoE metric type including latency, throughput, jitter, etc. |
| Event Reporting information | M | Information indicates how the notification is supposed to be sent, threshold based or the notification is periodical or the Immediate reporting is requested |
| >Threshold | O | Threshold of QoE metrics |
| >Reporting period | O | Indicating the metrics reporting period |
| >Immediate reporting flag | O | Indicating the request needs immediate reporting or not |
| Proposed expiration time | O | Proposed expiration time for the subscribe |

#### 9.4.3.6	QoE metrics response

Table 9.4.3.6-1 describes the information elements for the QoE metrics response from the VAL server to the NSCE server.

Table 9.4.3.6-1: QoE metrics Response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (NOTE) | O | Indicates that the QoE metrics request was successful. |
| > Subscription ID | M | Subscription identifier corresponding to the subscription. |
| >QoE metrics report | O | List of result values for the observed or computed QoE metrics value if the immediate reporting is needed. |
| Failure response (NOTE) | O | Indicates that the QoE metrics request failed. |
| > Cause | O | Indicates the cause of QoE metrics request failure |
| NOTE: One of these IEs shall be present in the message. |  |  |

#### 9.4.3.7	QoE metrics notification

Table 9.4.3.7-1 describes the information elements for the QoE metrics notification from the VAL server to the NSCE server.

Table 9.4.3.7-1: QoE metrics Notify

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Indicates that the QoE metrics request was successful. |
| QoE metrics report | M | List of result values for the observed or computed QoE metrics value. |

#### 9.4.3.8	Network slice LCM recommendation request

Table 9.4.3.8-1 describes information elements for the Network slice LCM recommendation request from the NSCE server to the VAL server.

Table 9.4.3.8-1: Network slice LCM recommendation Request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Identifier | M | Unique identifier of the requestor (i.e. NSCE server ID). |
| Security credentials | M | Security credentials resulting from a successful authorization. |
| Network slice LCM recommendation | M | Recommended network slice lifecycle management operation |
| >Network slice identifier(i.e. S-NSSAI) | M | Unique identifier of network slice (i.e. S-NSSAI) |
| >Recommend network slice LCM action | M | Recommend network slice LCM action(i.e. modifying the configuration, allocating a network slice) |
| >Network slice information | O | Network slice information if the action is taken(i.e. NEST) |

#### 9.4.3.9	Network slice LCM recommendation response

Table 9.4.3.9-1 describes the information elements for the Network slice LCM recommendation response from the VAL server to the NSCE server.

Table 9.4.3.9-1: Network slice LCM recommendation Response

| Information element | Status | Description |
| --- | --- | --- |
| Recommendation accepted flag | M | Indicates that the Network slice LCM recommendation request was accepted or not. |

### 9.4.4	APIs

#### 9.4.4.1	General

Table 9.4.4.1-1 illustrates the API for application layer network slice lifecycle management.

Table 9.4.4.1-1: API for application layer network slice lifecycle management

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Ensce_AppLayerNSLCM | Subscribe | Subscribe/Response | VAL server |
|  | Notify | Notify | VAL server |
| Eval_QoEMetrics | Subscribe | Subscribe/Response | NSCE |
|  | Notify | Notify | NSCE |
| Eval_NSLCMRecommendation | Subscribe | Subscribe/Response | NSCE |

#### 9.4.4.2	Ensce_AppLayerNSLCM_Subscribe operation

API operation name: Ensce_AppLayerNSLCM_Request

Description: The consumer subscribes for AppLayer_NS_LCM.

Inputs: See clause 9.4.3.2.

Outputs: See clause 9.4.3.3.

See clause 9.4.2 for details of usage of this operation.

#### 9.4.4.3	Ensce_AppLayerNSLCM_Notify operation

API operation name: Ensce_AppLayerNSLCM_Notify

Description: The consumer is notified with result of AppLayer_NS_LCM.

Inputs: See clause 9.4.3.4.

Outputs: None

See clause 9.4.2 for details of usage of this operation.

#### 9.4.4.4	Eval_QoEMetrics_Subscribe operation

API operation name: Eval_QoEMetrics_Subscribe

Description: The consumer subscribes for the QoE metrics.

Inputs: See clause 9.4.3.5.

Outputs: See clause 9.4.3.6.

See clause 9.4.2 for details of usage of this operation.

#### 9.4.4.5	Eval_QoEMetrics_Notify operation

API operation name: Eval_QoEMetrics_Notify

Description: The consumer is notified with the QoE metrics.

Inputs: See clause 9.4.3.7.

Outputs: None

See clause 9.4.2 for details of usage of this operation.

#### 9.4.4.6	Eval_NSLCMRecommendation _Request operation

API operation name: Eval_NSLCMRecommendation _Request

Description: The consumer request for whether to take the recommendation of the AppLayer_NS_LCM.

Inputs: See clause 9.4.3.8.

Outputs: See clause 9.4.3.9.

See clause 9.4.2 for details of usage of this operation.

## 9.5	Network slice optimization based on AF policy

### 9.5.1	General

Based on policy from the vertical applications (e.g., to trigger some network slice operations when the pre-configured thresholds are met), the network slice parameters for the vertical applications could be optimized by triggering the slice modification. The slice optimization can be triggered by OAM, NWDAF, and NSCE server itself as describe in 9.5.2.2.

### 9.5.2	Procedure

#### 9.5.2.1		AF policy provisioning

Figure 9.5.2.1 illustrates the procedure of AF policy provisioning from VAL server to NSCE server.

Pre-conditions:

1.	The NSCE server has information about the existing slice/slice profile/slice services which VAL server is using。

Figure 9.5.2.1: AF policy provisioning

1.	VAL server sends AF policy provisioning request to NSCE server. The request contains the policy, VAL server ID and S-NSSAI. Optionally, the request contains the indicator of policy harmonization.

The AF policy is in form of a policy profile which contains list of trigger events associated with the parameters and expected actions.

The supported policies are:

-	Based on monitored performance metric from OAM, when the max number of PDU sessions or max number of UE is reached, trigger the slice modification with expected parameters.

-	Based on monitored Network Slice load predictions from NWDAF, when Network Slice load predictions (Predicted Number of PDU Session establishments at the Network Slice) exceeds the threshold with high confidence, trigger the slice modification with expected parameters.

-	Based on the monitored the time period, when getting to a certain time period (e.g. summer vacation, spring festival etc.), trigger the slice modification with expected parameters.

2.	The NSCE server checks whether the policy is conflict with the service profile and other pre-configured policy. One criterion is to translate the network slice parameters in the service profile to see whether it is conflict with that in the AF provided policy. If yes, the request could be rejected. The NSCE server also checks the validity of the policy (in terms of time, or number of times the event has occurred) to avoid a ping-pong effect of slice modification. If the policy is invalid, the request could be rejected.

3.	If the policy harmonization is requested, the NSCE server may determine parameters harmonizing the policy if previously authorized..

Editor's note: The detail procedure of the policy harmonization is FFS.

4.	NSCE server sends the AF policy provisioning response to the VAL server to indicating whether the request is successful or not. If it is successful, policy ID is provided to VAL server.

#### 9.5.2.2		Network slice optimization based on AF policy

Figure 9.5.2.2 illustrates the procedure of network slice optimization based on AF policy.

Pre-conditions:

1.	The NSCE server is authorized to get network slice management data notification from OAM, and/or NWDAF;

2.	The VAL server is authorized to the NSCE server for network slice optimization.

3.	There is enough network capacity when the expected action is to expand the network slice.

4.	The AF policy has been pre-configured on the VAL server.

5.	The AF policy has been provided to the NSCE server as specified in clause 9.5.2.1.

Figure 9.5.2.2: Network slice optimization based on AF policy

1.	VAL server sends network slice optimization subscription request to NSCE server. The request contains the policy ID indicating the different policies.

2.	The NSCE server translates the trigger event to service API(s) with necessary parameters, and subscribe to the related service if needed.

-	To get the monitored performance metric from OAM, the notifyThresholdCrossing as defined in TS 28.532[7] clause 11.3.1.3 which is filled in with corresponding S-NSSAI in objectInstance could be used.

-	To monitor the Network Slice load predictions from NWDAF, the NSCE server subscribe to the NWDAF prediction by using the Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request as defined in TS 23.288[4] clause 6.1.1, and the procedures are defined in TS 23.288[4] clause 6.3.4, and clause 6.8.

-	To monitor the time period, the NSCE server setup the timer.

3.	NSCE server sends the network slice optimization subscription response to the VAL server to confirm the subscription of network slice optimization.

4.	Upon receiving the notification which indicating the threshold in the trigger event is met or specific time period is arrived, the NSCE server performs the expected action by triggering the slice modification as specified in the AF policy. The network slice modification could be triggered by consuming the Network Slice Provisioning service with the modifyMOIAttributes operation as specified in TS 28.531 [8]. The OAM responds back to NSCE server that the requested slice modification was successful or not. The slice modification requests contain the parameters need to be updated, including at least one of the following, uLThptPerSlice, dLThptPerSlice, maximum number of UEs, maximum number of PDU session as specified in TS 28.541 [z].

NOTE 1:	The slice modification could be done by application layer network slice lifecycle management as defined in clause 9.4.

5.	The NSCE server provides a network slice optimization notification to the VAL server.

NOTE 2:	There is no expectation to have constant and exact mapping between slice configuration parameters and actual traffic load of the same slice.

### 9.5.3	Information flows

#### 9.5.3.1	General

The following information flows are specified:

-	AF policy provisioning request and notification;

-	Network slice optimization subscription, response and notification.

#### 9.5.3.2	AF policy provisioning request

Table 9.5.3.2-1 describes information elements for the AF policy provisioning request from the VAL server to the NSCE server.

Table 9.5.3.2-1: AF policy provisioning Request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Identifier | M | Unique identifier of the requestor (i.e. VAL server ID). |
| Security credentials | M | Security credentials resulting from a successful authorization. |
| Requested slice information | O | Indication of the slice which is requested. |
| Requested DNN | O | Indication of the DNN which is requested. |
| Indicator of policy harmonization | O | Indicating whether the policy harmonization is requested. |
| Policy | O | The policy profile is defined in Table 9.5.3.2-2. The supported AF policies are listed in Table 9.5.3.2-3 to Table 9.5.3.2-5. |

Table 9.5.3.2-2 describes Policy profile of the AF policy provisioning request.

Table 9.5.3.2-2: Policy profile

| Information element | Status | Description |
| --- | --- | --- |
| Policy | O | The name of AF policy. |
| Trigger event | M | Indicating the event that should be monitored, associated with the threshold of the monitored parameter. |
| Expected action | M | Indicating the excepted actions associated with the updated parameter. |
| Lifetime or number of events | M | Time duration or number of events for which the policy is valid. |

Editor's note: Whether more policies consisting of trigger events and actions could be supported is FFS.

Table 9.5.3.2-3 to Table 9.5.3.2-5 list the supported policies.

Table 9.5.3.2-3: Policy of Max number of PDU sessions/ max number of UEs

| Information element | Status | Description |
| --- | --- | --- |
| Policy | O | Max number of PDU sessions/ max number of UEs |
| >Trigger event | M | Threshold PDU sessions/ Threshold UEs (reached utilization of available capacity in %) |
| >Expected action | M | Modification of PDU sessions / max number of UEs (step for increase in %) |
| Lifetime or number of events | M | Time duration |

Table 9.5.3.2-4: Policy of Network slice load prediction

| Information element | Status | Description |
| --- | --- | --- |
| Policy | O | Network slice load prediction |
| >Trigger event | M | Network Slice load predictions from NWDAF as defined in TS 23.288[4] clause 6.1.1 (exceeding utilization of available capacity in %) |
| >Expected action | M | Modification of related network slice parameters (step for increase in %) |
| Lifetime or number of events | M | Time duration |

Table 9.5.3.2-5: Policy of Time period

| Information element | Status | Description |
| --- | --- | --- |
| Policy | O | Time period |
| >Trigger event | M | Time/day configuration where specific network slice capacity is expected |
| >Expected action | M | Modification of slice capacity (increase or decrease in %) |
| Lifetime or number of events | M | Time duration |

#### 9.5.3.3	AF policy provisioning response

Table 9.5.3.3-1 describes the information elements for the AF policy provisioning response from the NSCE server to the VAL server.

Table 9.5.3.3-1: AF policy provisioning Response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (NOTE) | O | Indicates that the AF policy provisioning request was successful. |
| > Policy ID | M | Identifies the provided policy. |
| Failure response (NOTE) | O | Indicates that the AF policy provisioning request failed. |
| > Cause | M | Indicates the cause of AF policy provisioning request failure. |
| NOTE: One of these IEs shall be present in the message. |  |  |

#### 9.5.3.4	Network slice optimization subscription request

Table 9.5.3.4-1 describes information elements for the Network slice optimization subscription request from the VAL server to the NSCE server.

Table 9.5.3.4-1: Network slice optimization subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Identifier | M | Unique identifier of the requestor (i.e. VAL server ID). |
| Security credentials | M | Security credentials resulting from a successful authorization for the NSCE service. |
| Notification Target Address | O | The Notification Target Address (e.g. URL) where the notifications destined for the requestor should be sent to. |
| Requested slice information | O | Indication of the slice which is requested. |
| Requested DNN | O | Indication of the DNN which is requested. |
| Policy ID | O | Identifies the AF Policy. |
| Proposed expiration time | O | Proposed expiration time for the subscription. |

#### 9.5.3.5	Network slice optimization subscription response

The information elements specified in the table 9.5.3.5-1 is used for the Network slice optimization subscription response sent from the NSCE server to the VAL server.

Table 9.5.3.5-1: Network slice optimization subscription Response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (NOTE) | O | Indicates that the Network slice optimization subscription request was successful. |
| > Subscribe ID | M | Identifies the Network slice optimization subscribe event. |
| Failure response (NOTE) | O | Indicates that the Network slice optimization subscription request failed. |
| > Cause | O | Indicates the cause of Network slice optimization subscription request failure |
| NOTE: One of these IEs shall be present in the message. |  |  |

#### 9.5.3.6	Network slice optimization notify

The information elements specified in the table 9.5.3.6-1 is used for the Network slice optimization notify sent from the NSCE server to the VAL server.

Table 9.5.3.6-1: Network slice optimization Notify

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (NOTE) | O | Indicates that the Network slice optimization request was successful. |
| > Subscribe ID | M | Identifies the Network slice optimization subscribe event. |
| >Network slice information | M | Network slice information (i.e. NEST) with network slice identifier(i.e. S-NSSAI) |
| Failure response (NOTE) | O | Indicates that the Network slice optimization request failed. |
| > Cause | O | Indicates the cause of Network slice optimization request failure |
| NOTE: One of these IEs shall be present in the message. |  |  |

### 9.5.4	APIs

#### 9.5.4.1	General

Table 9.5.4.1-1 illustrates the API for Network slice optimization based on AF policy.

Table 9.5.4.1-1: Network slice optimization based on AF policy

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Eval_AFPolicyProvisioning | Request | Request/Response | NSCE |
| Ensce_NSOptimization | Subscribe | Subscribe/Notify | VAL |

#### 9.5.4.2	Eval_AFPolicyProvisioning operation

API operation name: Eval_AFPolicyProvisioning_Request

Description: The consumer subscribes for AF Policy Provisioning.

Inputs: See clause 9.5.3.2.

Outputs: See clause 9.5.3.3.

See clause 9.5.2.1 for details of usage of this operation.

#### 9.5.4.6	Ensce_NSOptimization_Subscribe Request operation

API operation name: Ensce_NSOptimization _ Subscribe

Description: The consumer request for Network Slice Optimization.

Inputs: See clause 9.5.3.4.

Outputs: See clause 9.5.3.5.

See clause 9.5.2.2 for details of usage of this operation.

#### 9.5.4.3	Ensce_NSOptimization_Notify operation

API operation name: Ensce_NSOptimization_Notify

Description: The consumer is notified with result of Network Slice Optimization.

Inputs: See clause 9.5.3.6.

Outputs: None

See clause 9.5.2.2 for details of usage of this operation.

## 9.6	Discovery of management service exposure

### 9.6.1	General

A Management Domain (MD) feature/capability is anything of use offered by the management system to the 3rd party application. Therefore, a new feature could be a managed entity, a MnS or management API, any software, hardware, or other functionality – for example, new technology support, new coverage area, new network slice type or instance, new NFs or new network slice subnet type or instance. To be able to utilise capabilities/features of the management system the applications must be made aware of existence of such features and capabilities. All MD features/capabilities come with a pre-configured exposure, where this can be configured by the operator for a given slice. This exposure is used to decide which application can see which information regarding the capability/feature. Exposure refers to the permissions that the 3rd party entity has gained over its use of the management service, e.g., the ability to read, or execute or modify or delete can be considered as different sorts of exposure.

The first procedure (9.6.2) provides the NSCE server support for translating the VAL server request to an MnS requirement and the exposure of service/management data related to his request. The second procedure (9.6.3) supports the notification to the VAL server, based on a new/modified MnS / capability for a target slice.

### 9.6.2	VAL-triggered MnS discovery procedure

In this procedure, the VAL server initially requests the new MnS which are supported by a target slice and based on this request the NSCE server requests from the OAM/MnS registry the MnS discovery. Then, the OAM/MnS registry derives the details to be exposed based on the NSCE server/VAL server permissions and provides the list of MnS for the given slice and the access details via the NSCE server to the VAL server.

Pre-conditions:

1.	The VAL server has registered to receive NSCE services

2.	The NSCE server is trusted by the OAM, and a contract has been signed between the MNO and NSCE server provider to allow OAM interfaces and/or network slice information exposure.

3.	MnS registry at OAM is aware of the allowed MnS and the exposure levels for a given slice

Figure 9.6.2-1 illustrates a solution for the MnS discovery support.

Figure 9.6.2-1: MnS discovery support

The steps of this procedure are as follows:

1.	The VAL server sends a Management Service Discovery subscribe request message to NSCE server to indicate a requirement for receiving the expected exposure capability type and the related exposure level/ permissions (e.g. permitted CRUD operations) for a target slice or for a given VAL application.

2.	The NSCE server sends a response to the subscription request, indicating a success or failure of the subscription request.

NOTE:	If CAPIF is used, the API interactions for the MnS discovery subscription shall be compliant with CAPIF as specified in 3GPP TS 23.222.

3.	The NSCE server translates the requirement based on the subscription, and in particular identifies the needed exposure capability type and translates it to an MnS exposure requirement. Such translation of the requirement includes the mapping to a list of MnSs (specified in TS 28.532) which are needed for the given slice and VAL server permissions.

4.	The NSCE server coordinates with the 5GS and discovers the related network service(s), by triggering the MnS service discovery as specified in TS 28.537[9], clause 5.2.1.3. In particular, NSCE server acts as MnS discovery service consumer (for all MnS needed for a target slice based on VAL requirement) requests from MnS discovery service producer(s) at OAM the discovery of MnS information for the MnS related to the target slice or VAL application. The 5GS, MnS discovery service producer(s) sends the discovery result including related exposure information such as management service identifier, management service information and management service producer information to the NSCE server.

Editor's Note:	The actual connectivity in step 4 (e.g., exposure of MnS via CAPIF or EGMF) is up to SA5 and FFS.

5.	The NSCE server sends a management service discovery subscription notify message to provide the service/management data based on step 4, including the list of MnS and the corresponding exposure details to the VAL server.

### 9.6.3	OAM-triggered new/modified MnS discovery

This procedure includes the case when a new/modified MnS is deployed at the MD for the given slice, and the OAM/MnS registry provides this information directly to VAL server (assuming that VAL server has registered to the MD).

Pre-conditions:

1.	VAL server is registered to NSCE server based on clause 9.2.

2.	NSCE server has subscribed to OAM / MnS discovery service registry

Figure 9.6.3-1: OAM-triggered new/modified MnS discovery

1.	The MnS registry may discover new or modified MnS in the 3GPP MD. Such changes may be due to new/modified management service producers, new managed entities (such as new radio, new technology or new NFs), new/modified technical support (such as support in a new geography or coverage area), availability of new management data, e.g., related to slice performance. An example procedure related to the addition of a new MnS producer is provided in TS 28.537 clause 5.2.1.1.

The MnS discovery service producer when capturing new or modified capability (e.g. new MnS producer) related to the target slice or VAL application identifies the NSCE server which needs to be informed (based on the subscription) on the new/modified capability and derives the details to be exposed to the VAL server (via the NSCE server). Then, the MnS registry sends to the NSCE server the new / modified MnS.

Editor's Note:	The actual connectivity in step 1 (e.g. exposure of MnS via CAPIF or EGMF) is up to SA5 and FFS.

2.	The NSCE server determines the VAL server(s) that needs to be notified on the new or modified MnS / MD capability based on their registrations. Then, NSCE server generates and sends a notification with the required information to be provided to the VAL server.

3.	The information on the discovered new/modified MnS is sent as a management service discovery notification message from the NSCE server to VAL server.

### 9.6.4	Information flows

#### 9.6.4.1	General

The following information flows are specified for the MnS discovery support based on 9.6.2 and 9.6.3.

#### 9.6.4.2	Management service discovery subscribe request

Table 9.6.4.2-1 describes information elements for the Management service discovery subscribe request from the VAL server to the NSCE server.

Table 9.6.4.2-1: Management service discovery subscribe request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | The identifier of the VAL server |
| VAL service ID | M | The identifier of the VAL service for which the MnS discovery request applies |
| Slice identifier | O | The slice identifier (S-NSSAI, NSI ID or ENSI) which is mapped to the VAL application, if known by the VAL server |
| Exposure capability requirement | O | The requirement includes indication on the needed exposure level / permissions for exposing information related to the target slice. Also, the requirement may include the exposure capability type which is supported (e.g. via EGMF or directly to MnS producer) |

#### 9.6.4.3	Management service discovery subscribe response

Table 9.6.4.3-1 describes information elements for the Management service discovery subscribe response from the NSCE server to the VAL server.

Table 9.6.4.3-1: Management service discovery subscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the subscription request (positive or negative acknowledgement) |

#### 9.6.4.4	Management service discovery notify

Table 9.6.4.4-1 describes information elements for the Management service discovery notify message from the NSCE server to the VAL server.

Table 9.6.4.4-1: Management service discovery notify

| Information element | Status | Description |
| --- | --- | --- |
| VAL service ID | M | The identifier of the VAL application |
| Management Domain/System ID | M | The identifier of the management system/domain of interest |
| List of MnS IDs / MnS producer IDs | M | The list of identifiers of the needed MnSs / MnS producers |
| >MnS capability | M | The capability per needed MnS. Such capability may related to the managed elements such as considerations for radio, technology, coverage or NFs |
| >MnS exposure level | O | The exposure level for the needed MnS. This level relates to the permissions of the VAL server over the MnS, e.g. whether to read, write, delete, update. |
| Slice identifier | O | The slice identifier (S-NSSAI, NSI ID or ENSI) which is mapped to the VAL application and the list of MnSs. |

### 9.6.5	APIs

Table 9.6.5-1 illustrates the NSCALE APIs for the MnS discovery feature.

Table 9.6.5-1: List of APIs for the MnS discovery feature

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_Management_Service_Discovery | Management_Service Discovery | VAL server | Subscribe/Notify |

#### 9.6.5.1	SS_ Management_Service Discovery

##### 9.6.5.1.1	General

API description: This API enables the VAL server to communicate with the network slice capability enablement server for requesting management service discovery over NSCE-S.

##### 9.6.5.1.2	Management_Service Discovery

API operation name: Management_Service Discovery

Description: Providing for Management_Service Discovery subscribe to the NSCE server and receiving a confirmation.

Known Consumers: VAL server.

Inputs: See subclause 9.6.2 (step 1)

Outputs: See subclause 9.6.2 (step 4), 9.6.3 (step 3)

## 9.7	Network slice related performance and analytics monitoring

### 9.7.1	General

The NSCE server supports the end to end network slice related performance and analytic monitoring capability exposure. The NSCE server identifies which data are needed, collects the data from different data sources (e.g., the OAM system, the Core Network or the VAL end users), performs the data organizations and aggregations and expose the processed data to VAL servers for monitoring. Following clauses are for the procedures, information flows and APIs for Network slice related performance and analytics monitoring request and Network slice related performance and analytics reporting respectively. The procedure in clause 9.7.2.1 is for task creation of the network slice performance and analytics monitoring. The procedure in clause 9.7.2.2 is for the performance and analytics data retrieving when the data is available.

### 9.7.2	Procedure

#### 9.7.2.1	Network slice related performance and analytics monitoring request

For network slice related performance and analytics monitoring capabilities exposed to the VAL server, the VAL server triggers the procedure by sending the network slice related performance and analytics monitoring request to NSCE server as Figure 9.7.2.2-1 shows:

Figure 9.7.2.1-1: Request Network slice related performance and analytics monitoring

1.	The VAL server sends a request to NSCE server to collect the desired service/VAL service specific performance data and analytics data, the detailed content of the reported data depends on the type of the VAL services. The required end-to-end network slice related data performance data are indicated by Perflist IE as described in table 9.7.3.2-1.

2.	The NSCE server shall check if the VAL server is authorized to request the the network slice performance and analytics data monitoring.

3.	NSCE server determines the requested data needed to collect from network side and collects the performance measurements and analytics data of network slice from 5GS. For OAM system, the APIs defined in clause 11.3, TS 28.532[7] is utilized, e.g., packet delay, radio resource utilization. For CN functions, the APIs of Nnwdaf_AnalyticsInfo service defined in clause 7.3, TS 23.288[4] is utilized, e.g., slice load level related network data analytics, slice load level related network data analytics.

4.	NSCE server retrieves the KQI data of services, the network performance related data and the end user’s information from NSCE client.

Editor's Note: The mechanism defined in EVEX study will be reused in Data collection from VAL users. The services and APIs to be utilized is FFS.

5.	NSCE server correlates and analyses the performance data of network slice instance, the analytics data of group of UEs and the KQI/QoE data to generate the performance data and analytics data report as required by VAL server.

6.	NSCE server responds to VAL server to inform the VAL server if the monitoring request is succeed.

#### 9.7.2.2	Network slice related performance and analytics report subscription and report

For network slice related performance and analytics result subscription and report, the VAL server triggers the procedure by sending the network slice related performance and analytics report subscription to NSCE server, the NSCE server reports the performance and analytics report after the subscription is succeed, as Figure 9.7.2.2-1 shows:

Figure 9.7.2.2-1: Network slice related performance and analytics report subscription and report

1.	The VAL server subscribes to the required performance and analytics report.

2.	The NSCE server response to the VAL server to indicate whether the report is successfully generated and ready.

3.	NSCE server sends the performance reports to VAL server if the report is ready.

### 9.7.3	Information flows

#### 9.7.3.1	General

The following information flows are specified for Network slice related performance and analytics monitoring:

-	Network slice related performance and analytics monitoring request and response

-	Network slice related performance and analytics monitoring report subscription

-	Network slice related performance and analytics monitoring report

#### 9.7.3.2	Network slice related performance and analytics monitoring request and response

Table 9.7.3.2-1 and Table 9.7.3.2-2 describe information elements for the network slice related performance and analytics monitoring request and response between the VAL server and the NSCE server.

Table 9.7.3.2-1: Network slice related performance and analytics monitoring request

| Information element | Status | Description |
| --- | --- | --- |
| VAL information | M | The information of the VAL server |
| Performance Monitoring Request ID | M | Identifier of the performance and analytics monitoring |
| Performance and analytics monitoring metrics | M | The information of performance and analytics monitoring |
| > VAL service identity | M | Identifier of the VAL service to be monitored |
| >PerfList | M | The list of performance to be monitored |
| >> PerfName | M | The name of the performance to be reported, e.g., the end to end round-trip time or the end to end network slice load. |
| >S-NSSAI | O | Identifier of the network slice to be monitored |
| >StartTime | M | The start time point of the performance and analytics monitoring |
| >EndTime | O | The end time point of the performance and analytics monitoring, If the EndTime IE is not included, it indicates that the performance and analytics monitoring will not stop until the monitoring request is released or updated. |

Table 9.7.3.2-2: Network slice related performance and analytics monitoring response

| Information element | Status | Description |
| --- | --- | --- |
| status | M | The result of performance and analytics monitoring request |
| >Successful response | CM (NOTE 1) | Indicates that the VAL server performance and analytics monitoring request was successful. |
| > > Performance Monitoring Request ID | M | Identifier of the performance and analytics monitoring |
| >>StartTime | M | The start time point of the performance and analytics monitoring |
| >>EndTime | O | The end time point of the performance and analytics monitoring, If the EndTime IE is not included, it indicates that the performance and analytics monitoring will not stop until the monitoring request is released or updated. |
| >Failure response | CM (NOTE 2) | Indicates that the performance and analytics monitoring request failed. |
| >>Cause | M | Indicates the cause of VAL performance and analytics monitoring request failure |
| NOTE 1: Information element shall be present when performance and analytics monitoring request is successful. NOTE 2 Information element shall be present when performance and analytics monitoring request status is failure. |  |  |

#### 9.7.3.3	Network slice related performance and analytics report subscription

Table 9.7.3.3-1 and 9.7.3.3-2 describe information elements for Network slice related performance and analytics report subscription from the NSCE server to the VAL server.

Table 9.7.3.3-1: Network slice related performance and analytics report subscription

| Information element | Status | Description |
| --- | --- | --- |
| VAL information | M | The information of the VAL server |
| >VAL server ID | M | The identifier of the VAL server |
| Report ID | M | Identifier of performance and analytics results the report |
| Report Information | M | The information of performance and analytics report retrieving |
| > VAL service identity | M | Identifier of the VAL service of which the performance and analytics results are required |
| >S-NSSAI | O | Identifier of the network slice of which the performance and analytics results are required |
| >TimeDuration |  | The time duration the report involved |
| >PerfList | M | The list of performance to be reported |
| >>PerfName | M | The name of the performance to be reported |

Table 9.7.3.3-2: Response of Network slice related performance and analytics report subscription

| Information element | Status | Description |
| --- | --- | --- |
| status | M | The result of performance and analytics subscription |
| >Successful response | CM (NOTE 1) | Indicates that the VAL server performance and analytics report subscription was successful. |
| >>Report ID | M | Identifier of the performance and analytics report Id |
| >Failure response | CM (NOTE 2) | Indicates that the performance and analytics subscription failed. |
| >>Cause | M | Indicates the cause of VAL performance and analytics subscription failure |
| NOTE 1: Information element shall be present when performance and analytics subscription is successful. NOTE 2 Information element shall be present when performance and analytics subscription status is failure. |  |  |

#### 9.7.3.4	Network slice related performance and analytics report

Table 9.7.3.4-1 and 9.7.3.4-2 describe information elements for Network slice related performance and analytics report from the NSCE server to the VAL server.

Table 9.7.3.4-1: Network slice related performance and analytics report request

| Information element | Status | Description |
| --- | --- | --- |
| VAL information | M | The information of the VAL server |
| >VAL server ID | M | The identifier of the VAL server |
| Report ID | M | Identifier of the report |

Table 9.7.3.4-2: Network slice related performance and analytics report

| Information element | Status | Description |
| --- | --- | --- |
| Report ID | M | Identifier of the report |
| PerfResultFile | CM (NOTE 1) | PerfResultFile contains one or more PerfResult |
| >PerfResult | CM | Information element containing the VAL service identity or S-NSSAI followed by a list of result values for the aggregated or analyzed network slice related performance |
| >VAL service identity | M | Identifier of the VAL service of which the performance and analytics results are reported |
| >S-NSSAI | O | Identier of the network slice of which the performance and analytics results are reported |
| >ResultsValueList |  | List of ResultsValue |
| >>ResultsValue | M | Information element containing the perfName and perfValue. |
| >>>PerfName | M | The name of the performance to be reported |
| >>>PerfValue | M | The corresponding value of the monitored performance |
| Failure response | CM (NOTE 2) | Indicates that network slice related performance and analytics results reporting failed. |
| >Cause | CM (NOTE 2) | Indicates the cause of network slice related performance and analytics results report failure |
| NOTE 1: Information element shall be present when the network slice related performance and analytics results retrieving is successful. NOTE 2: Information element shall be present when the network slice related performance and analytics results retrieving is failure. |  |  |

### 9.7.4	APIs

#### 9.7.4.1	General

Table 9.7.4.1-1 and 9.7.4.1-2 illustrate the API for network slice related performance and analytics monitoring.

Table 9.7.4.1-1: Ensce_PerfMonitoring API

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Ensce_PerfMonitoring | Perf_Monitoring_Request | Request /Response | VAL server |

Table 9.7.4.1-2 Ensce_PerfResultReport API

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Ensce_PerfReportSubscription | Perf_ PerfReport_Subscription | Subscription/response | VAL Server |

Table 9.7.4.1-3: Ensce_PerfReport API

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Ensce_PerfReporting | Perf_Report | Request /Response | VAL Server |

#### 9.7.4.2	Ensce_PerfMonitoring API

API operation name: Perf_Monitoring

Description: The consumer requests to monitor network slice related performance and analytics.

Known Consumers: VAL server.

Inputs: See table 9.7.3.2-1.

Outputs: See table 9.7.2.2-2.

See clause 9.7.2.1 for details of usage of this operation.

#### 9.7.4.3	Ensce_ PerfReportSubscription API

API operation name: Perf_Result_Reporting

Description: The consumer requests to report network slice related performance and analytics.

Known Consumers: VAL server.

Inputs: See table 9.7.3.3-1.

Outputs: See table 9.7.3.3-2.

See clause 9.7.2.2 for details of usage of this operation.

#### 9.7.4.4	Ensce_ PerfReport API

API operation name: Perf_ Report

Description: The consumer requests to report network slice related performance and analytics.

Known Consumers: VAL server.

Inputs: See table 9.7.3.4-1.

Outputs: See table 9.7.3.4-2.

See clause 9.7.2.2 for details of usage of this operation.

## 9.8	Information collection from NSCE server(s)

### 9.8.1	General

The collected information by NSCE server could be exposed to other NSCE server(s) to optimize network resource allocation, based on the agreement among the NSCE servers. Then the NSCE server providing network slice related information acts as the producer, while the NSCE server obtaining network slice related information is the consumer.

Editor's note: The business relationship and the deployment model among these NSCE servers are FFS.

### 9.8.2	Procedure

#### 9.8.2.1	Information collection from NSCE server(s) subscribe request and response

Pre-condition:

1.	The producer NSCE server #2 has agreement with consumer NSCE server #1 to provide the collected slice information.

# Figure 9.8.2.1-1: Information collection from NSCE server(s) subscribe request and response

1.	The NSCE server#1 sends out the information collection subscribe request with expected period and interested slice ID, e.g., List of SNSSAI. This step could be done by pre-configuration.

2.	The NSCE server #2 shall check if the NSCE server #1 is authorized to get the network slice information.

3.	After authentication, the NSCE server #2 collects slice information.

Editor's note: The service Network slice related performance and analytics monitoring could be re-utilized to collected slice information. While the details and reference of Network slice related performance and analytics monitoring is FFS.

Editor's note: The behaviour of slice information collection in the case NSCE server or VAL server request this from an edge NCSE server is FFS.

4.	The NSCE server #2 sends collected slice information to NSCE server #1.

5.	The NSCE server #1 may process the collected slice information (such as information aggregation), if needed.

#### 9.8.2.2	Information collection from NSCE server(s) Notify

# Figure 9.8.2.2-1: Information collection from NSCE server(s) Notify

1.	Once the reporting period is reached or the threshold is crossed, the NSCE server #2 sends information collection notify to NSCE server #1.

### 9.8.3	Information flows

#### 9.8.3.1	General

The following information flows are specified for Information collection from NSCE server(s):

-	Information collection from NSCE server(s) subscribe request, response and notification;

#### 9.8.3.2	Information collection from NSCE server(s) subscribe request

Table 9.8.3.2-1 describes information elements for the Information collection from NSCE server(s) subscribe request from the consumer NSCE server to the producer NSCE server(s).

Table 9.8.3.2-1: Information collection from NSCE server(s) subscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identifier | M | Unique identifier of the requester (i.e. NSCE server ID). |
| Security credentials | M | Security credentials resulting from a successful authorization for the NSCE service. |
| Notification Target Address | O | The Notification Target Address (e.g. URL) where the notifications destined for the requester should be sent to. |
| List of S-NSSAI(s) | M | Identifier of the interested network slice |
| >QoS type indicator | O | QoS metric type including latency, throughput, jitter, etc. |
| >Threshold | O | Threshold of QoS metrics |
| >Reporting period | O | Indicating the metrics reporting period |
| >Immediate reporting flag | O | Indicating the request needs immediate reporting or not |
| Proposed expiration time | O | Proposed expiration time for the subscription |

#### 9.8.3.3	Information collection from NSCE server(s) subscribe response

The information elements specified in the Table 9.8.3.3-1 is used for the Information collection from NSCE server(s) subscribe response sent from the producer NSCE server to the consumer NSCE server.

Table 9.8.3.3-1: Information collection from NSCE server(s) subscribe Response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (NOTE 1) | O | Indicates that the request was successful. |
| > Subscription ID | M | Subscription identifier corresponding to the subscription. |
| >Network slice related performance and analytics report | O (NOTE 2) | Network slice performance and analytics (i.e. slice load) with network slice identifier(i.e. S-NSSAI) |
| Failure response (NOTE 1) | O | Indicates that the request failed. |
| > Cause | O | Indicates the cause of request failure |
| NOTE 1: One of these IEs shall be present in the message. NOTE 2: When the Immediate reporting is requested, this IE shall be present. |  |  |

Editor's note: The detail of Network slice related performance and analytics report is FFS.

#### 9.8.3.4	Information collection from NSCE server(s) notify

The information elements specified in Table 9.8.3.4-1 are used for the information collection from NSCE server(s) notify sent from the producer NSCE server to the consumer NSCE server.

Table 9.8.3.4-1: Information collection from NSCE server(s) Notify

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription. |
| >Network slice related performance and analytics report | M | Network slice performance and analytics (i.e. slice load) with network slice identifier(i.e. S-NSSAI) |

### 9.8.4	APIs

#### 9.8.4.1	General

Table 9.8.4.1-1 illustrates the API for information collection from NSCE server(s).

Table 9.8.4.1-1: API for Information collection from NSCE server(s)

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| SSnsce_InfoCollection | Subscribe | Subscribe/Response | NSCE server |
|  | Notify | Notify | NSCE server |

#### 9.8.4.2	SSnsce_InfoCollection_Subscribe operation

API operation name: SSnsce_InfoCollection_Subscribe

Description: The consumer subscribes for Information collection from NSCE server(s).

Inputs: See clause 9.8.3.2.

Outputs: See clause 9.8.3.3.

See clause 9.8.2.1 for details of usage of this operation.

#### 9.8.4.3	SSnsce_InfoCollection_Notify operation

API operation name: SSnsce_InfoCollection_Notify

Description: The consumer is notified with result of information collection from NSCE server(s).

Inputs: None.

Outputs: See clause 9.8.3.4.

See clause 9.8.2.2 for details of usage of this operation.

## 9.x	<procedure name>

Add a copy of this clause for each procedure, adding a title, a general description, information flows and the procedure. Include an APIs clause if APIs are needed for the procedure.

### 9.x.1	General

### 9.x.2	Procedure

### 9.x.3	Information flows

### 9.x.4	APIs (if applicable)

This clause may be skipped if the procedure under 10.x does not require specification of APIs.

Annex A (informative):
Deployment models

Informative annexes may appear in both Technical Specifications and Technical Reports. Use style "Heading 8" for use in TSs.

Informative annexes shall not contain requirements for the implementation of the Technical Specification.

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-06 | SA6#49-bis-e |  |  |  |  | TS skeleton | 0.0.0 |
| 2022-07 | SA6#49-bis-e |  |  |  |  | TS Skeleton agreed in SA6#49-bis-e: S6-221640, Implemented pCRs approved in SA6#49-bis-e: S6-221641, S6-221642, S6-221643, S6-221822, S6-221818. | 0.1.0 |
| 2022-09 | SA6#50-e |  |  |  |  | Implemented pCRs approved in SA6#50: S6-222146, S6-222197, S6-222407 Editorial changes by the rapporteur. | 0.2.0 |
| 2022-10 | SA6#51-e |  |  |  |  | Implemented pCRs approved in SA6#51: S6-223036, S6-223035, S6-222901, S6-222764. Editorial changes by the rapporteur. | 0.3.0 |
| 2022-11 | SA6#52 |  |  |  |  | Implemented pCRs approved in SA6#51: S6-223449, S6-223517, S6-223595, S6-223557, S6-223207, S6-223477 Editorial changes by the rapporteur. | 0.4.0 |