| 3GPP TS 23.540 V0.3.0 (2021-12) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; 5G System; Technical realization of Service Based Short Message Service Stage 2 (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	7

3.3	Abbreviations	7

4	Architecture to support SBI-based SMS	7

4.1	Architecture to support SBI-based SMS	7

4.2	Reference point to support SBI-based SMS	9

4.3	Service based interface to support SBI-based SMS	9

5	Procedures for SBI-based SMS	9

5.1	Procedure for SBI-based MT SMS	9

5.1.1	General	9

5.1.2	Successful Mobile Terminated short message transfer without SMS Router/ IP-SM-GW	10

5.1.3	Successful Mobile Terminated short message transfer via SMS Router	11

5.1.4	Successful Mobile Terminated short message transfer via IP-SM-GW	13

5.1.5	Unsuccessful Mobile Terminated short message transfer without SMS Router/IP-SM-GW	15

5.2	Procedure for SBI-based MO SMS	16

5.2.1	General	16

5.2.2	Procedure for Successful Mobile Originated short message transfer	16

5.2.3	Unsuccessful Mobile Originated short message transfer	17

6	Services for SBI-based SMS	18

6.1	General	18

6.2	Nudm_ReportSMDeliveryStatus service	19

6.2.1	Nudm_ReportSMDeliveryStatus_Request service operation	19

6.3	Niwmsc_SMService service	19

6.3.1	General	19

6.3.2	Niwmsc_SMService_MoForwardSm service operation	19

6.4	Nsmsf_SMService service	19

6.4.1	General	19

6.4.2	Nsmsf_SMService_MtForwardSm service operation	19

6.5	Nipsmgw_SMService service	20

6.5.1	General	20

6.5.2	Nipsmgw_SMService_RoutingInfo service operation	20

6.5.3	Nipsmgw_SMService_MtForwardSm service operation	20

6.6	Nrouter_SMService service	20

6.6.1	General	20

6.6.2	Nrouter_SMService_RoutingInfo service operation	20

6.6.3	Nrouter_SMService_MtForwardSm service operation	20

Annex <X> (informative): Change history	22

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In drafting the TS/TR, pay particular attention to the use of modal auxiliary verbs! TRs shall not contain any normative provisions.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

This clause is optional. If it exists, it shall be the second unnumbered clause.

# 1	Scope

The present document defines the Stage 2 architecture, procedures and services to support service based short message service (SMS) in 5G system (5GS).

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[3]	3GPP TS 23.501: "System architecture for the 5G System (5GS); Stage 2".

[4]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[5]	3GPP TS 23.632: "User data interworking, coexistence and migration; Stage 2".

[6]	3GPP TS 29.540: "5G System; SMS Services; Stage 3".

[7]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[8]	3GPP TS 24.011: "Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface".

[9]	3GPP TS 29.573: "5G System; Public Land Mobile Network (PLMN) Interconnection; Stage 3".

[10]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

alert-SC: service element provided by a GSM/UMTS/EPS/5GS PLMN to inform an SC which has previously initiated unsuccessful short message delivery attempt(s) to a specific MS/UE, that the MS/UE is now recognized by the PLMN to have recovered operation.

Gateway MSC For Short Message Service (SMS-GMSC): function of an MSC capable of receiving a short message from an SC, interrogating an HLR/HSS/UDM for routing information and SMS info, and delivering the short message to the VMSC/SGSN/MME/SMSF of the recipient MS/UE.

Interworking MSC For Short Message Service (SMS-IWMSC): function of an MSC capable of receiving a short message from within the PLMN and submitting it to the recipient SC.

IP-Short-Message-Gateway (IP-SM-GW): function responsible for protocol interworking between the IP-based UE and the SC.

Service Centre (SC): function responsible for the relaying and store and forwarding of a short message between an SME and an MS/UE.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

GPSI	Generic Public Subscription Identifier

MO SMS	Mobile Originated Short Message Service

MT SMS	Mobile Terminated Short Message Service

NAS	Non-Access-Stratum

NP	Number Portability

NRF	Network Repository Function

SBA	Service Based Architecture

SBI	Service Based Interface

SMS	Short Message Service

SM MO	Short Message Mobile Originated

SM MT	Short Message Mobile Terminated

SMSF	Short Message Service Function

SMSoNAS	SMS over NAS

UDM	Unified Data Management

# 4	Architecture to support SBI-based SMS

## 4.1	Architecture to support SBI-based SMS

Figure 4.1-1 shows the non-roaming architecture to support SBI-based SMS.

Figure 4.1-1: Non-roaming System Architecture for SBI-based SMS

Figure 4.1-2a and Figure 4.1-2a depict the non-roaming architecture to support SBI-based SMS, using the reference point representation showing how various network functions interact with each other

Figure 4.1-2a: Non-roaming System Architecture for SBI-based MT SMS in reference point representation

Figure 4.1-2b: Non-roaming System Architecture for SBI-based MO SMS in reference point representation

NOTE 1:	The newly introduced reference points for SMS_SBI are marked in red, and numbered from 1 to 10,

Editor's Note:	The reference points 1 to 10 should be further numbered like other reference points in 5GS (e.g. N7, N8 and so on).

## 4.2	Reference point to support SBI-based SMS

Besides the reference point to support SMS over NAS described in clause 4.4.2.2 of TS 23.501 [3], the following reference points are needed to support SBI-based SMS:

Reference point 1:	Reference point for routing information between SMS-GMSC and UDM.

Reference point 2:	Reference point between SMS-IWMSC and UDM.

Reference point 3:	Reference point for routing information between IP-SM-GW and UDM.

Reference point 4:	Reference point for routing information between SMS Router and UDM.

Reference point 5:	Reference point for SMS message transfer between SMS-GMSC and SMSF.

Reference point 6:	Reference point for SMS message transfer between SMS-GMSC and SMS Router.

Reference point 7:	Reference point for SMS message transfer between SMSF and SMS Router.

Reference point 8:	Reference point for SMS message transfer between SMS-GMSC and IP-SM-GW.

Reference point 9:	Reference point for SMS message transfer between SMSF and IP-SM-GW

Reference point 10:	Reference point for SMS message transfer between SMS-IWMSC and SMSF.

## 4.3	Service based interface to support SBI-based SMS

Besides the service based interfaces to support SMS over NAS described in clause 4.4.2.3 of TS 23.501 [3], the following service based interfaces are needed to support SBI-based SMS.

Nudm:	Service-based interface exhibited by UDM

Nnrf:	Service-based interface exhibited by NRF

Ngmsc:	Service-based interface exhibited by SMS-GMSC

Niwmsc:	Service-based interface exhibited by SMS-IWMSC

Nipsmgw:	Service-based interface exhibited by IP-SM-GW

Nrouter:	Service-based interface exhibited by SMS Router

# 5	Procedures for SBI-based SMS

This clause depicts procedures for MT SMS, MO SMS and other related services.

## 5.1	Procedure for SBI-based MT SMS

### 5.1.1	General

The procedure is used to support the Mobile Terminated short message transfer for SBI-based interfaces as defined in clause 4.1, which is based on Short message mobile terminated procedure defined in clause 10.1 of 3GPP TS 23.040 [2] and clauses 4.13.3.6, 4.13.3.7 and 4.13.3.8 of 3GPP TS 23.502 [4].

This new services introduced in this procedure can be registered in NRF, and discovered by NF service consumers.

### 5.1.2	Successful Mobile Terminated short message transfer without SMS Router/ IP-SM-GW

Figure 5.1.2-1: MT SMS over NAS without SMS Router/ IP-SM-GW

1.	MT SMS interaction between SC and SMS-GMSC follow the current procedure as defined in 3GPP TS 23.040 [2].

Editor's Note:	SBI services of the MNP NF to derive the subscription network from the GPSI is FFS.

2a-2b.	The SMS-GMSC should query the NRF to find the UDM instance that manages the user subscriptions using the GPSI.

3.	SMS-GMSC invokes Nudm_SmsRoutingInfo_Request (GPSI) to the UDM to get the serving SMSF instance for UE.

4.	The UDM shall check the registration/reachability flags to determine the potential target nodes and responds to the SMS-GMSC by sending Nudm_SmsRoutingInfo_Get response, in this procedure the SMSF instance Id is included in the response message.

Editor's Note:	Whether the new service in step 3-4 is needed or the existing Nudm_UECM_Get service operation can be reused is FFS.

5.	The SMS-GMSC forwards the SMS message to the SMSF by invoking Nsmsf_SMService_MtForwardSm service operation.

6.	MT SMS over NAS procedure between SMSF, AMF and UE is same as the definition in step 4a to 6b of Figure 4.13.3.6-1 of 3GPP TS 23.502 [4].

7.	The SMSF delivers the delivery report to SMS-GMSC by sending the Nsmsf_SMService_MtForwardSm response to the SMS-GMSC.

8.	The SMS-GMSC updates the SM-Delivery Report Status to UDM by invoking Nudm_SMReportStatus_Request.

9.	UDM responses Nudm_SMReportStatus_Request response to SMS-GMSC.

10.	The SMS-GMSC delivers the delivery report to SC as defined in TS 23.040 [2].

11.	MT SMS over NAS procedure between SMSF, AMF and UE is same as the definition in step 6c to 6d of Figure 4.13.3.6-1 of 3GPP TS 23.502 [4].

### 5.1.3	Successful Mobile Terminated short message transfer via SMS Router

Figure 5.1.3-1: MT SMS over NAS via SMS Router

1.	MT SMS interaction between SC and SMS-GMSC follow the current procedure as defined in 3GPP TS 23.040 [2].

2a-2b.	The SMS-GMSC should query the NRF to find the UDM instance that manages the user subscriptions using the GPSI.

3.	SMS-GMSC invokes Nudm_SmsRoutingInfo_Request (GPSI) to the UDM to get the serving node information for UE.

4.	The UDM shall check the registration/reachability flags to determine the potential target nodes, e.g. SMSF. For MT SM transfer via SMS Router, the UDM shall invoke the Nrouter_SMService_RoutingInfo to provide the SMSF Instance Id to the SMS Router. The address of the SMS Router to be contacted by the UDM may be configured locally.

5.	The SMS Router shall send Nrouter_SMService_RoutingInfo response to the UDM.

6.	UDM responds to the SMS-GMSC by sending Nudm_SmsRoutingInfo_Request response (SMS Router address).

Editor's Note:	Whether the new service in step 3 and 6 is needed or the existing Nudm_UECM_Get service operation can be reused is FFS.

7.	The SMS-GMSC forwards the SMS message to the SMS Router by invoking Nrouter_SMService_MtForwardSm service operation

8.	The SMS Router forwards the SMS message to the SMSF by invoking Nsmsf_SMService_MtForwardSm service operation.

9.	MT SMS over NAS procedure between SMSF, AMF and UE is same as the definition in step 4a to 6b of Figure 4.13.3.6-1 in 3GPP TS 23.502 [4].

10.	The SMSF delivers the delivery report to SMS Router by sending the Nsmsf_SMService_MtForwardSm response to the SMS Router.

11.	The SMS Router delivers the delivery report to SMS-GMSC by sending the Nrouter_SMService_MtForwardSm response to the SMS-GMSC.

12.	The SMS Router may report the SM-Delivery Status to UDM by invoking Nudm_ReportSMDeliveryStatus_Request.

13.	UDM responses Nudm_ReportSMDeliveryStatus_Request response to SMS Router.

14-15.	The SMS-GMSC may report the SM-Delivery Status to the UDM by invoking Nudm_ReportSMDeliveryStatus_Request and the UDM shall ignore the information provided in this report.

16.	The SMS-GMSC delivers the delivery report to SC as defined in 3GPP TS 23.040 [2].

17.	MT SMS over NAS procedure between SMSF, AMF and UE is same as the definition in step 6c to 6d of Figure 4.13.3.6-1 in 3GPP TS 23.502 [4].

Editor's Note:	SBI services of the MNP NF to derive the subscription network from the GPSI is FFS.

### 5.1.4	Successful Mobile Terminated short message transfer via IP-SM-GW

Figure 5.1.4-1: MT SMS over NAS via IP-SM-GW

1.	MT SMS interaction between SC and SMS-GMSC follow the current procedure as defined in 3GPP TS 23.040 [2].

2a-2b.	The SMS-GMSC should query the NRF to find the UDM instance that manages the user subscriptions using the GPSI.

3.	The SMS-GMSC invokes Nudm_SmsRoutingInfo_Request (GPSI) to the UDM to get the serving node information for the UE.

4.	The UDM shall check the registration/reachability flags to determine the potential target nodes, e.g. SMSF. For MT SM transfer via IP-SM-GW, the UDM shall invoke the Nipsmgw_SMService_RoutingInfo to provide the SMSF Instance Id to the IP-SM-GW. The address of the IP-SM-GW to be contacted by the UDM may be configured locally.

5.	The IP-SM-GW shall send Nipsmgw_SMService_RoutingInfo response to the UDM.

6.	The UDM responds to the SMS-GMSC by sending Nudm_SmsRoutingInfo_Request response (IP-SM-GW address).

Editor's Note:	Whether the new service in step 3 and 6 is needed or the existing Nudm_UECM_Get service operation can be reused is FFS.

7.	The SMS-GMSC forwards the SMS message to the IP-SM-GW by invoking Nipsmgw_SMService_MtForwardSm service operation

8.	The IP-SM-GW forwards the SMS message to the SMSF by invoking Nsmsf_SMService_MtForwardSm service operation.

9.	The MT SMS over NAS procedure between SMSF, AMF and UE is the same as in step 4a to 6b of Figure 4.13.3.6-1 of 3GPP TS 23.502 [4].

10.	The SMSF delivers the delivery report to the IP-SM-GW by sending the Nsmsf_SMService_MtForwardSm response to the IP-SM-GW.

11.	The IP-SM-GW delivers the delivery report to the SMS-GMSC by sending the Nipsmgw_SMService_MtForwardSm response to the SMS-GMSC.

12.	The IP-SM-GW may report the SM-Delivery Status to the UDM by invoking Nudm_ReportSMDeliveryStatus_Request.

13.	The UDM responses with Nudm_ReportSMDeliveryStatus_Request response to the IP-SM-GW.

14-15.	The SMS-GMSC may report the SM-Delivery Status to the UDM by invoking Nudm_ReportSMDeliveryStatus_Request and the UDM shall ignore the information provided in this report.

16.	The SMS-GMSC delivers the delivery report to the SC as defined in 3GPP TS 23.040 [2].

17.	The MT SMS over NAS procedure between SMSF, AMF and UE is the same as in step 6c to 6d of Figure 4.13.3.6-1 of 3GPP TS 23.502 [4].

Editor's Note:	SBI services of the MNP NF to derive the subscription network from the GPSI is FFS.

### 5.1.5	Unsuccessful Mobile Terminated short message transfer without SMS Router/IP-SM-GW

Figure 5.1.5-1: Unsuccessful MT SMS over NAS without SMS Router/ IP-SM-GW

1.	MT SMS interaction between SC and SMS-GMSC follow the current procedure as defined in 3GPP TS 23.040 [2].

2a-2b.	The SMS-GMSC should query the NRF to find the UDM instance that manages the user subscriptions using the GPSI. If there is no available UDM returned from NRF, the SMS-GMSC may send error response to the SC.

3.	SMS-GMSC invokes Nudm_SmsRoutingInfo_Request (GPSI) to the UDM to get the serving SMSF instance for UE.

4.	The UDM shall check the registration/reachability flags to determine the potential target nodes. If the UDM is failed at this step, e.g. user not found in the UDM, the UDM shall respond to the SMS-GMSC by sending Nudm_SmsRoutingInfo_request response with error cause.

Editor's Note:	Whether the new service in step 3-4 is needed or the existing Nudm_UECM_Get service operation can be reused is FFS.

5.	If successful response is returned in step 4, the SMS-GMSC forwards the SMS message to the SMSF by invoking Nsmsf_SMService_MtForwardSm service operation.

6.	MT SMS over NAS procedure between SMSF, AMF and UE is same as the definition in step 4a to 6b of Figure 4.13.3.6-1 of 3GPP TS 23.502 [4].

7.	If the AMF informs the SMSF that it cannot deliver the MT-SMS to the UE in step 6, e.g. UE is not reachable, or the SMSF is failed at this step, e.g. memory capacity exceeded, the SMSF shall send the Nsmsf_SMService_MtForwardSm response with error cause to the SMS-GMSC.

8.	The SMS-GMSC may report the SM-Delivery Status (e.g. UE is not reachable or memory capacity exceeded) to UDM by invoking Nudm_ReportSMDeliveryStatus_Request. After receiving the report from the SMS-GMSC, the UDM shall set the reachability flag, update the MWD and should subscribe the reachability report in the AMF.

9.	UDM responses Nudm_ReportSMDeliveryStatus_Request response to SMS-GMSC.

10.	The SMS-GMSC sends the failure report to SC as defined in TS 23.040 [2].

## 5.2	Procedure for SBI-based MO SMS

### 5.2.1	General

The procedure for SBI-based MO SMS is showed in Figure 5.2.2-1, which is based on MO SMS procedures in 3GPP TS 23.040 [2] clause 10.2. Compared to procedures in 3GPP TS 23.040 [2], new services are introduced in, including:

-	Niwmsc_SMService service provided by SMS-IWMSC.

This new service is registered in NRF, and can be invoked by service consumers.

### 5.2.2	Procedure for Successful Mobile Originated short message transfer

Figure 5.2.2-1: Procedures for successful SBI-based SM MO message transfer

0	SMS-IWMSC registers Niwmsc_SMService service in the NRF, during the NF registration procedure.

1	MO SM message transfer from UE to SMSF through AMF follows the current procedure as defined in 3GPP TS 23.040 [2]

2a	If SMSF knows from local configuration that the target SMS-IWMSC does not support SBI, it shall quit the SBI-based procedure and fallback to legacy (MAP/Diameter) protocol based procedures, as defined in TS 23.040 [2],

or SMSF invokes the Nnrf_NFDiscovery to discover and select serving SMS-IWMSC with the parameters of SUPI and/or GPSI and/or location (e.g. TAIs, CGIs, etc.) and/or E.164 address of the SC.

2b	If no SMS-IWMSC could be discovered, the NRF shall feedback to SMSF, and SMSF shall quit the SBI-based procedure and fallback to legacy (MAP/Diameter) protocol based procedures, as defined in TS 23.040 [2].

If a SMS-IWMSC is discovered and selected, NRF returns the IP addresses or FQDNs of the serving SMS-IWMSC to provide Niwmsc_SMService service to SMSF.

3	SMSF sends a Niwmsc_SMService_MoForwardSm service request to the URI of serving SMS-IWMSC, which is obtained in step 2b. The payload body of the request shall contain the SM record to be sent, the Service Centre address, the callbackURI for MO SMS Delivery Report, the timer for waiting the MO SMS Delivery Report, and optionally contains the Access Type.

4	MO SMS delivery procedure between SMS-IWMSC and SC is the same as the definition in step 4 of Figure 4.13.3.3-1 of 3GPP TS 23.502 [4].

5	SMS-IWMSC sends Niwmsc_SMService_MoForwardSm response to deliver the MO SMS delivery report to the URI of serving SMSF, which is obtained in step 3

6	MO SMS delivery report procedure between SMSF, AMF and UE is the same as the 3GPP TS 23.502 [4].

When no more SMS is to be sent, the procedure for CP-ack and SMS ack is the same as the 3GPP TS 23.502 [4].

These procedures are defined in step 6a to 6d of Figure 4.13.3.3-1 of 3GPP TS 23.502 [4].

### 5.2.3	Unsuccessful Mobile Originated short message transfer

Figure 5.2.3-1 depicts procedure for unsuccessful SBI-based SM MO message transfer

Figure 5.2.3-1: Procedures for unsuccessful SBI-based SM MO message transfer

0-3	The same as the procedures in step 0-3 of Figure 5.2.2-1 in clause 5.2.2

4	SMS-IWMSC sends Niwmsc_SMService_MoForwardSm response with HTTP status code for application errors as defined in Table 5.3.2-1.

5	Failure report from SMSF to UE, with the error cause code as defined in Table 5.3.2-2.

SMS-IWMSC indicates the different errors for MO SM transfer in MoForwardSm response according to the different failure scenarios which happened during MO SM transfer. The Application Errors used in Niwmsc_SMService_MoForwardSm response are defined in Table 5.3.2-1 below

Table 5.3.2-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| UNKNOWN_SERVICE_CENTRE_ADDRESS | 403 Forbidden | The delivery of the MO short message failed because SMS-SC was unknown. |
| SERVICE_CENTRE_CONGESTION | 403 Forbidden | The delivery of the MO short message failed because SMS-SC was in congestion. |
| USER_NOT_SERVICE_CENTER | 403 Forbidden | The delivery of the short message failed because the user didn't belongs to the SMS-SC. |
| FACILITY_NOT_SUPPORTED | 403 Forbidden | The delivery of the MO short message failed because of facility not supported. |
| INVALID_SME_ADDRESS | 403 Forbidden | The delivery of the MO short message failed because the SME address is invalid. |
| UNREACHABLE_SMS_SC | 504 Gateway Timeout | The delivery of the MO short message failed because the response is timeout. |

If errors are indicated by the SMS-IWMSC, the SMSF shall send a failure report (i.e. a RP-ERROR message) to the UE, with the error cause coded as following map between errors indicated by SMS-IWMSC and error cause code in RP-ERROR message:

Table 5.3.2-2: MAP of Application errors

| Return error from SMS-IWMSC | Cause value in the RP-ERROR message |
| --- | --- |
| The Response from SMS-IWMSC is timeout Unspecified 4xx/5xx error code | 38 Network out of order |
| 403 Forbidden with FACILITY_NOT_SUPPORTED | 69 Requested facility not implemented |
| 403 Forbidden with UNKNOWN_SERVICE_CENTRE_ADDRESS | 1 Unassigned number |
| 403 Forbidden with SERVICE_CENTRE__CONGESTION | 42 Congestion |
| 403 Forbidden with USER_NOT_SERVICE_CENTER | 28 Unidentified subscriber |
| 403 Forbidden with INVALID_SME_ADDRESS | 21 Short message transfer rejected |

NOTE:	The coding and the use of the RP-ERROR message is specified in 3GPP TS 24.011 [8].

# 6	Services for SBI-based SMS

## 6.1	General

The following table illustrates the services for SBI-based SMS.

Table 7.1-1: Services for SBI-based SMS

| Service Name | Service Operations | Operation Semantics | Service Privider(s) | Service Consumer(s) |
| --- | --- | --- | --- | --- |
| SmsRoutingInfo | Request | request / response | UDM | SMS-GMSC |
| ReportSMDeliveryStatus | Request | request / response | UDM | SMS-GMSC, IP-SM-GW, SMS Router |
| SMService | MtForwardSm | request / response | SMSF | SMS-GMSC, IP-SM-GW, SMS Router |
|  | MoForwardSm | request / response | SMS-IWMSC | SMSF |
|  | RoutingInfo | request / response | IP-SM-GW, SMS Router | UDM |
|  | MtForwardSm | request / response | IP-SM-GW, SMS Router | SMS-GMSC |
| NOTE: |  |  |  |  |

## 6.2	Nudm_ReportSMDeliveryStatus service

### 6.2.1	Nudm_ReportSMDeliveryStatus_Request service operation

Service operation name: Nudm_ReportSMDeliveryStatus_Request

Description: reports the SM-Delivery Status to UDM.

Inputs, Required: GPSI, SM-Delivery status

Inputs, Optional: None.

Outputs, Required: report SM-Delivery status result.

Outputs, Optional: None.

## 6.3	Niwmsc_SMService service

### 6.3.1	General

Service Description: This service can be used for SBI-based MO SM transmit and Delivery Report through SMS-IWMSC.

### 6.3.2	Niwmsc_SMService_MoForwardSm service operation

Service operation name: Niwmsc_SMService_MoForwardSm.

Description: Service request from consumer to SMS-IWMSC for MO SM transmit.

Inputs, Required: the SM record to be sent, the Service Centre address, the callbackURI for MO SMS Delivery Report, the timer for waiting MO SMS Delivery Report.

Inputs, Optional: Access Type.

Outputs, Required: On success, Delivery Report should be returned.

On failure or redirection, the appropriate HTTP status code (e.g. "403 Forbidden", "504 Gateway Timeout") indicating the error shall be returned.

Outputs, Optional: None.

## 6.4	Nsmsf_SMService service

### 6.4.1	General

This service is defined in clause 5.2.9.2 of 3GPP TS 23.502 [4], in addition, following new service operations shall be supported.

### 6.4.2	Nsmsf_SMService_MtForwardSm service operation

Service operation name: Nsmsf_SMService_MtForwardSm

Description: transmit downlink SMS message from consumer NF to SMSF.

Inputs, Required: GPSI, SMS payload

Inputs, Optional: None.

Outputs, Required: SMS message transmission result.

Outputs, Optional: None.

## 6.5	Nipsmgw_SMService service

### 6.5.1	General

Service Description: This service can be used for SBI-based MT SM transmit through IP-SM-GW.

### 6.5.2	Nipsmgw_SMService_RoutingInfo service operation

Service operation name: Nipsmgw_SMService_RoutingInfo

Description: provides the SMSF Instance Id to the IP-SM-GW.

Inputs, Required: GPSI, SMSF Instance Id.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

### 6.5.3	Nipsmgw_SMService_MtForwardSm service operation

Service operation name: Nipsmgw_SMService_MtForwardSm

Description: transmit downlink SMS message from consumer NF to IP-SM-GW and return the MT SMS Delivery Report to the consumer NF.

Inputs, Required: GPSI, SMS payload

Inputs, Optional: None.

Outputs, Required: MT SMS Delivery Report.

Outputs, Optional: None.

## 6.6	Nrouter_SMService service

### 6.6.1	General

Service Description: This service can be used for SBI-based MT SM transmit through SMS Router.

### 6.6.2	Nrouter_SMService_RoutingInfo service operation

Service operation name: Nrouter_SMService_RoutingInfo

Description: provides the SMSF Instance Id to the SMS Router.

Inputs, Required: GPSI, SMSF Instance Id.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

### 6.6.3	Nrouter_SMService_MtForwardSm service operation

Service operation name: Nrouter_SMService_MtForwardSm

Description: transmit downlink SMS message from consumer NF to SMS Router and return the MT SMS Delivery Report to the consumer NF.

Inputs, Required: GPSI, SMS payload

Inputs, Optional: None.

Outputs, Required: MT SMS Delivery Report.

Outputs, Optional: None.

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-10 | CT4#106e | C4-215046 |  |  |  | C4-215046 as basis | 0.1.0 |
| 2021-10 | CT4#106e | C4-215513 |  |  |  | Implementation of C4-215417, C4-215418, C4-215453 and C4-215454 in CT4#106e | 0.2.0 |
| 2021-12 | CT4#107e | C4-216466 |  |  |  | Implementation of C4-216442, C4-216443, C4-216444 and C4-216546 in CT4#107e | 0.3.0 |