| 3GPP TS 23.632 V16.1.0 (2020-03) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; User data interworking, coexistence and migration; Stage 2; (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	7

4	System architecture	7

4.1	Architecture for direct UDM-HSS interworking	7

4.2	Reference points for direct UDM-HSS interworking	8

4.3	Service based interfaces for direct UDM-HSS interworking	8

4.4	Subscription Identifiers	8

4.5	HSS Discovery and Selection	9

4.6	UDM Discovery and Selection	9

4.7	Subscription Profiles	9

5	System procedures	10

5.1	General	10

5.2	Authentication	10

5.2.1	General	10

5.2.2	Vector Generation in HSS	10

5.2.3	Vector Generation in UDM/ARPF	12

5.2.4	HSS using the Nudr SBI	13

5.3	5GC-EPC Mobility Scenarios	14

5.3.1	General	14

5.3.2	Mobility from 5GC to EPC, with N26	14

5.3.3	Mobility from EPC to 5GC, with N26	15

5.4	Scenarios of Interworking with IMS	21

5.4.1	T-ADS	21

5.4.2	P-CSCF Restoration	22

5.4.3	Network Provided Location Information Request	23

5.4.4	User State Retrieval	25

5.4.5	UE Reachability	26

5.4.6	IMEI Retrieval	28

5.4.7	SRVCC: IMS AS obtaining SRVCC data	29

5.4.8	SRVCC: IMS AS obtaining SRVCC data – HSS using Nudr	30

5.4.9	SRVCC: IMS AS updating STN-SR	30

5.5	SMS Support	31

5.5.1	General	31

5.5.2	MT-SMS Routing Information Retrieval	31

5.5.3	MT-SMS Delivery Failure	33

5.5.4	SMS Alerting	34

5.5.5	MT-SMS Routing Information Retrieval Over Nudr	36

6	Network Function Service procedures	36

6.1	HSS Services	36

6.1.1	General	36

6.1.2	Nhss_UEAuthentication service	37

6.1.2.1	Nhss_UEAuthentication_Get service operation	37

6.1.3	Nhss_UECM service	37

6.1.3.1	Nhss_UECM_MMEDeregistration service operation	37

6.1.3.2	Nhss_UECM_Update service operation	37

6.1.4	Nhss_SDM service	37

6.1.4.2	Nhss_SDM_Notification service operation	38

6.1.4.3	Nhss_SDM_Subscribe service operation	38

6.1.4.4	Nhss_SDM_Unsubscribe service operation	38

6.2	UDM Services	38

6.2.1	General	38

6.2.2	Nudm_UECM service operation	39

6.2.2.1	Nudm_UECM_P-CscfRestorationTrigger service operation	39

6.2.2.2	Nudm_UECM_Get service operation	39

6.2.2.3	Nudm_UECM_AMFDeregistration service operation	39

6.2.2.4	Nudm_UECM_Update service operation	39

6.2.3	Nudm_MT Service	40

6.2.3.1	Nudm_MT_ProvideDomainSelectionInfo Service	40

6.2.3.2	Nudm_MT_ProvideUserState service operation	40

6.2.3.3	Nudm_MT_ProvideLocationInfo service operation	40

6.2.3.4	Nudm_MT_Provide5GSRVCCInfo Operation	40

6.2.4	Nudm_EE Service	41

6.2.4.1	Nudm_EventExposure_Subscribe service operation	41

6.2.4.2	Nudm_EventExposure_Notify service operation	41

6.2.5	Nudm_UEAuthentication service	41

6.2.5.1	Nudm_UEAuthentication_GetHssAv service operation	41

6.2.6	Nudm_SDM service operations	41

6.2.6.1	Nudm_SDM_Get service operation	41

6.2.6.2	Nudm_SDM_Subscribe service operation	41

6.2.6.3	Nudm_SDM_Unsubscribe service operation	41

6.2.6.4	Nudm_SDM_Notify service operation	41

6.2.7	Nudm_PP service operations	41

6.2.7.1	Nudm_PP_Update service operation	41

6.3	UDR Services	42

Annex A (informative): Change history	43

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document defines the Stage 2 architecture, procedures, flows and Network Function Services for User Data Interworking, Coexistence and Migration within the 5G System.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.002: "Network Architecture".

[4]	3GPP TS 23.380: "IMS Restoration Procedures".

[5]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[6]	3GPP TS 33.501: "Security Architecture and Procedures for 5G System".

[7]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[8]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE); Security architecture".

[9]	3GPP TS 33.402: "3GPP System Architecture Evolution (SAE); Security aspects of non-3GPP accesses".

[10]	3GPP TS 33.203: "3G security; Access security for IP-based services".

[11]	3GPP TS 33.220: "3G security; Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[12]	3GPP TS 24.080: "Mobile radio interface layer 3 supplementary services specification - Formats and coding".

[13]	3GPP TS 23.237: "IP Multimedia Subsystem (IMS) Service Continuity".

[14]	3GPP TS 23.008: "Organization of subscriber data".

[15]	3GPP TS 29.328: "IP Multimedia (IM) Subsystem Sh interface; Signalling flows and message contents".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

Void

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GS UDR	5G Unified Data Repository

EPS UDR	EPS User Data Repository

# 4	System architecture

## 4.1	Architecture for direct UDM-HSS interworking

Figure 4.1-1 shows the reference architecture for direct UDM-HSS interworking.

Figure 4.1-1: Architecture for Direct UDM-HSS interworking

Figure 4.1-2 shows the reference architecture for direct UDM-HSS interworking using the reference point representation.

Figure 4.1-2: Architecture for Direct UDM-HSS interworking in reference point representation

The 5GS-UDR (Unified Data Repository) and EPS-UDR (User Data Repository) may be collocated, forming a common repository.

NOTE:	The HSS is only using the NU2 reference point for the purpose of interworking with 5GS, i.e. NU2 is not a replacement of the Ud interface.

### 4.2	Reference points for direct UDM-HSS interworking

The following reference points are realized by service-based interfaces:

NU1:	Reference point between the HSS and the UDM.

NU2:	Reference point between the HSS and the 5GS-UDR.

For a list of other SBA reference points supported in 5GC see 3GPP TS 23.501 [2].

For a list of IMS reference points, including SBA IMS reference points, see 3GPP TS 23.228 [7].

For a list of Non-SBA reference points and Network Nodes interfacing the HSS see 3GPP TS 23.002 [3].

The HSS makes use of the Ud reference point to interact with the EPS-UDR. The UDM makes use of the N35 reference point to interact with the 5GS-UDR.

### 4.3	Service based interfaces for direct UDM-HSS interworking

Nudm:	Service-based interface exhibited by UDM.

Nhss:	Service-based interface exhibited by HSS.

The HSS shall make use of Nudm services as described in clause 6.2 and may make use of Nudr services as described in clause 6.3.

The UDM shall make use of Nhss services and Nudr services as described in clauses 6.1 and 6.3.

## 4.4	Subscription Identifiers

As defined in 3GPP TS 23.501 [2], for interworking with the EPC, the SUPI allocated to the 3GPP UE shall always be based on an IMSI to enable the UE to present an IMSI to the EPC.

The subscription identifier used over NU1 reference point in Nhss services shall be an IMSI. The UDM extracts the IMSI from the user's SUPI.

The subscription identifier used over NU1 reference point in Nudm services shall be a SUPI based on an IMSI. The HSS creates a SUPI from the user's IMSI or the IMSI associated to the user's public identifier in the EPS or IMS domain (e.g. user's MSISDN or IMPU).

## 4.5	HSS Discovery and Selection

This clause defines the procedures for HSS discovery and selection by the UDM. The procedures for HSS discovery and selection by SBI capable IMS entities is defined in 3GPP TS 23.228 [7].

The UDM performs HSS discovery to discover an HSS that manages the user subscriptions in EPC.

The UDM shall utilize the NRF to discover the HSS instance(s) unless the information about HSS instances is available by other means, e.g. locally configured on the UDM. The HSS selection function in UDM selects an HSS instance based on the available HSS instances (obtained from the NRF or locally configured).

When the NRF is used for HSS discovery, the HSS registers in the NRF using the Nnrf_NFManagement_NFRegister Request message as defined in 3GPP TS 23.502 [5].

Different HSS instances managing different sets of IMSI/MSISDN ranges may be deployed in a given PLMN. In this case, the HSS instances register in NRF using either different ranges of IMSI/MSISDN and/or HSS Group IDs.

NOTE 1:	In deployments where simple IMSI/MSISDN ranges are not suitable to describe the IMSI/MSISDN sets served by HSS instances, it is expected the HSS instances only register HSS Group IDs.

When NRF is used for HSS discovery, the UDM sends a Nnrf_NFDiscovery_Request to NRF as defined in 3GPP TS 23.502 [5] to discover HSS instances within a given PLMN. The UDM may store all returned HSS instances and their NF profiles for subsequent use, including, if applicable, supported IMSI/MSISDN ranges, and/or HSS Group IDs.

The UDM may use Nnrf_NFStatusSubscribe/Unsubscribe service operations with NRF as defined in 3GPP TS 23.502 [5] to receive Nnrf_NFStatusNotify service operation for updates to the NF profiles of HSS instances registered in NRF.

The UDM always selects an HSS within its own PLMN. The HSS selection should consider one of the following factors when available to the UDM entity:

1.	HSS Group ID of the UE's IMSI.

2.	IMSI; the UDM selects an HSS instance based on the IMSI range the UE's IMSI belongs to or based on the results of a discovery procedure with NRF using the UE's IMSI as input for HSS discovery.

NOTE 2:	In this release of the specification there is not identified need for the UDM to be able to select the HSS based on IMS identifiers (IMPI, IMPU) neither based on MSISDN or External Group ID.

## 4.6	UDM Discovery and Selection

The HSS performs UDM discovery and selection as described in 3GPP TS 23.501 [2].

## 4.7	Subscription Profiles

In the context of Mobility, IMS and SMS use cases, the HSS initiates interworking with the UDM only for UEs which have a 5G subscription that is known to be active in 5GC.

Note:	The HSS may be aware that a 5G subscription for the UE exists based on the Core Network Type restrictions defined for the UE (i.e. If restriction for Core Network Type indicates that the UE can access to 5GC, it implies that the UE has 5G subscription data).
Additionally, the HSS may be aware that a UE for which 5G subscription data exist is not active in 5GC e.g. when the HSS has not yet received an Nhss_UECM_MMEDeregistration request from the UDM for that UE (see clause 5.3.2 step 3).

# 5	System procedures

## 5.1	General

Procedures involving communication between HSS and UDM comprise Authentication, Mobilty, IMS interworking, and SMS support.

## 5.2	Authentication

### 5.2.1	General

A subscriber's authentication subscription data, including the subscriber's long-term key(s) and sequence number, shall be stored in a single repository so that a single sequence number can be maintained for the subscriber.

The subscriber's long-term key(s) shall not be transferred over the NU1 reference point between HSS and UDM. Therefore,  subscriber's authentication vectors for subscribers with both 5G and EPS subscriptions shall be calculated in one single place in the system which is either the UDM/ARPF or the HSS. Subscription data stored in the 5GS-UDR and EPS-UDR or alternatively local configuration in the UDM and HSS indicates which option is to be used for a given subscriber.

The following clauses specify the system procedures for these different alternatives.

### 5.2.2	Vector Generation in HSS

This clause specifies the procedures for authentication vector request when the subscriber's authentication subscription data is stored at the EPS-UDR. In this case, the UDM requests the generation of the Authentication Vector for 5GS to the HSS.

NOTE:	The HSS acts as ARPF rather than as AuC and it is required to generate authentication vectors as defined in 3GPP TS 33.501 [6].

Editor's Note: The possibility to collocate the ARPF in HSS (i.e. not within UDM as described in TS 33.501 [6]) needs to be confirmed by SA3.

When the UDM receives an authentication information Request from the AUSF it shall check (by means of an 5GS-UDR query or local configuration in the UDM) whether the subscribed authentication method is 5G_AKA or EAP_AKA_PRIME and if so whether 5G authentication vector generation for the identified subscriber shall be done in the HSS. If so, the UDM shall make use of the Nhss_UEAuthentication_Get service operation to retrieve a 5G authentication vector from the HSS.

Figure 5.2.2-1 shows the scenario where the authentication vector request for a 5G subscriber who also has an EPS subscription is received by the UDM.

Figure 5.2.2-1: Authentication for 5G subscriber with authentication vector generation in HSS

1.	The UDM receives an Authentication Vector request, containing the identity of the user (SUPI or SUCI). If SUCI is received, the UDM performs SUCI to SUPI de-concealment. For details of the Nudm_UEAuthentication Service see 3GPP TS 23.502 [5] and 3GPP TS 33.501 [6].

Editor's Note:	According to TS 33.501 [6], the ARPF holds the home network private key that is used by the SIDF to deconceal the SUCI and reconstruct the SUPI.  It needs to be confirmed if UDM would still be able to deconceal the SUPI in the case that the ARPF is colocated with the HSS rather than with the UDM.

2.	If the 5GS-UDR is used, the UDM queries the 5GS-UDR using the SUPI to retrieve Authentication Subscription Information. In this scenario the Authentication Subscription Information contains a subscribed authentication method of 5G_AKA or EAP_AKA_PRIME and an indicator indicating that authentication vector generation shall be performed in the HSS. Optionally, the indication that the authentication vector generation shall be performed in the HSS could be locally configured at the UDM/ARPF.

3.	The UDM uses the Nhss_UEAuthentication_Get service operation to retrieve an authentication vector from the HSS. The request contains the IMSI the authentication method and serving network name.

4.	The HSS reads authentication subscription data from the EPS-UDR. This step is omitted if all relevant authentication subscription data are stored locally in the HSS.

5.	The HSS (AuC/ARPF) calculates the requested authentication vector taking into account the serving network name and authentication method received in step 3 and the authentication subscription information retrieved from the EPS-UDR.

6.	The calculated authentication vector is sent to the UDM.

7.	The HSS updates the EPS-UDR with the new sequence number. This step is omitted if the sequence number is stored locally in the HSS.

8.	The UDM forwards the authentication vector to the AUSF.

### 5.2.3	Vector Generation in UDM/ARPF

This clause specifies the procedures for authentication vector request when the subscriber's authentication subscription data is stored at the 5GS-UDR. In this case, the HSS requests the generation of the Authentication Vector for EPS and/or IMS to the UDM.

NOTE:	The UDM acts as AuC rather than as ARPF and it is required to generate authentication vectors as defined in 3GPP TS 33.401 [8], 3GPP TS 33.402 [9], 3GPP TS 33.203 [10] and 3GPP TS 33.220 [11].

When the HSS receives an authentication vector request from a serving node (e.g. MME, SGSN, VLR, S-CSCF, BSF) it shall check (by means of an EPC-UDR query) whether authentication vector generation for the identified subscriber shall be done in the UDM. If so, the HSS shall make use of the Nudm_UEAuthentication GetHssAv service operation to retrieve authentication vectors from the UDM.

Figure 4.2.3-1 shows the scenario where an authentication vector request for a subscriber is received by the HSS and subscription data stored in the EPS-UDR indicate that for the subscriber authentication vector generation is to be performed in the UDM.

Figure 5.2.3-1: Authentication for a subscriber with authentication vector generation in UDM

1.	The HSS receives an Authentication Vector request, containing the identity of the user (IMSI, or Public User Identity and/or Private User Identity).

2.	The HSS queries the EPS-UDR using the identity of the user to retrieve Authentication Subscription Information. In this scenario the Authentication Subscription Information contains an indicator indicating that authentication vector generation shall be performed in the UDM.

3.	The HSS uses the Nudm_UEAuthentication_GetHssAv service operation to retrieve an authentication vector from the UDM. The request contains the identity of the user, the type of the requested vector (E-UTRAN/UTRAN or GERAN/ IMS-AKA and when available the visited PLMN-ID.

4.	The UDM reads authentication subscription data from the 5GS-UDR.

5.	The UDM (ARPF) calculates the requested authentication vectors taking into account the information received in step 3 and the authentication subscription information retrieved from the 5GS-UDR.

6.	The calculated authentication vectors are sent to the HSS.

7.	The UDM updates the 5GS-UDR with the new sequence number.

8.	The HSS forwards the authentication vectors to the serving node.

### 5.2.4	HSS using the Nudr SBI

When the HSS receives an S6a-AIR from the MME, it may check (by means of an EPC-UDR query) whether the subscriber has an 5G subscription. If so, the HSS can use of the Nudr_DM_Query Get service operation to retrieve the authentication subscription data from the 5GS UDR and generate the authentication vector.

Figure 5.2.4-1 shows the scenario where the authentication vector request for a 5G subscriber who also has an EPS subscription is received by the UDM.

Figure 5.2.4-1: Authentication for 5G subscriber with EPS subscription

1.	The HSS receives an Authentication Vector request containing the identity of the user (IMSI).

2.	The HSS queries the 5GS-UDR using the IMSI to retrieve Authentication Subscription Information. Since the subscriber is a 5G subscriber the response indicates that the subscriber's authentication information is stored in the 5GS UDR.

NOTE:	Local configuration in the HSS may indicate that authentication subscription data for all subscribers can be obtained from the 5G UDR and thus this step may be omitted.

3.	The HSS uses the Nudr_DM_Query Get service operation to retrieve the authentication subscription data from the 5GS UDR. The request contains the IMSI formatted as a SUPI.

4.	The HSS (AuC) calculates the requested authentication vector taking into account the serving network name and authentication method received in step 1 and the authentication subscription information retrieved from the 5GS-UDR in step 3.

5.	The calculated authentication vector is returned to the MME.

6.	The HSS updates the 5GS-UDR with the new sequence number.

## 5.3	5GC-EPC Mobility Scenarios

### 5.3.1	General

As defined in 3GPP TS 23.501 [2], when interworking procedures with N26 are used, the UE operates in single-registration mode. For the 3GPP access, either the AMF or the MME is registered in the HSS+UDM. This implies that:

-	The registration of an MME in HSS for a UE that is capable to access the 5GC, triggers the cancellation of the AMF address for 3GPP access registered in the UDM, if any.-	The registration of an AMF in UDM for a UE that is capable to access the EPC, triggers the cancellation of the MME address registered in the HSS, if any

Clauses 5.3.2 and 5.3.3 shows the interaction between the HSS and UDM to trigger the cancelation of the registration of the old serving node in the old access domain in an interworking scenario with N26.

NOTE:	Interworking procedures without N26 do not require that the HSS+UDM cancels the location of the old serving node in the old access domain.

Additionally, IP address preservation for PDU sessions that support interworking is provided to UEs during inter-system mobility procedures by storing and fetching the PGW-C+SMF FQDN for S5/S8 interface per APN/DNN via the HSS+UDM.

Clause 5.3.4 shows the interaction between the HSS and UDM for the fetching of the PGW-C+SMF FQDN.

### 5.3.2	Mobility from 5GC to EPC, with N26

Figure 5.3.2-1 shows the interaction between the HSS and UDM in an interworking scenario with N26, when the UE attaches to the EPC.

Figure 5.3.2-1: Mobility from 5GC to EPC

1.	The HSS receives an S6a ULR request containing the IMSI of the subscriber.

The HSS stores the new MME address. If the EPC UDR is used, the HSS updates the EPS-UDR with the new MME address and reads the subscription information related to the IMSI from the EPS-UDR.

The UE's subscription may include restriction for Core Network Type (5GC). If restriction for Core Network Type indicates that the UE can access to 5GC, it implies that the UE has 5G subscription data.

2.	The HSS responds to the MME with an S6a-ULA.

3.	If the subscription information related to the IMSI indicates that the UE has 5G subscription data, then the HSS uses the Nudm_UECM_AMFDeregistration service operation to request the UDM to cancel the registration of the user in 5GC for 3GPP access, if any. The HSS includes the user's IMSI received in step 1 as the user's SUPI in this request.

4.	The UDM responds to the HSS.

The UDM checks if the user is registered in 5GC over 3GPP access. The UDM may use the 5GS-UDR to retrieve the address of the AMF for 3GPP access registered in UDM for that user, if any. If the user is registered at an AMF for 3GPP access, step 5 is executed.

5.	If there is an AMF address for 3GPP access found for the user, the UDM cancels the user registration in 5GC by sending a Nudm_UECM_DeregistrationNotification to the AMF.

### 5.3.3	Mobility from EPC to 5GC, with N26

Figure 5.3.3-1 shows the interaction between the UDM and the HSS in an interworking scenario with N26, when the UE attaches to the 5GC.

Figure 5.3.3-1: Mobility from EPC to 5GC

1.	The UDM receives a Nudm_UECM_Registration request for 3GPP access containing the SUPI of the subscriber. In this case the SUPI shall be based on an IMSI.

The UDM stores the new AMF address for 3GPP access. If the 5GS UDR is used, the UDM updates the 5GS-UDR with the new AMF address for 3GPP access and reads the subscription information related to the SUPI from the 5GS-UDR.

The UE's subscription may include restriction for Core Network Type (EPC). If restriction for Core Network Type indicates that the UE can access to EPC, it implies that the UE has EPC subscription data.

2.	The UDM responds to the AMF with a Nudm_UECM_Registration Response.

3.	If the subscription information related to the SUPI indicates that the UE has EPC subscription data, then the UDM uses the Nhss_UECM_MMEDeregistration service operation to request the HSS to cancel the registration of the user in EPC, if any. The UDM includes the IMSI based SUPI received in step 1 as the user's SUPI in this request.

4.	The HSS responds to the UDM.

The HSS checks if the user is registered in EPC. The HSS may use the EPS-UDR to retrieve the address of the MME registered in HSS for that user, if any. If the user is registered at an MME, step 5 is executed.

5.	If there is an MME address found for the user, the HSS cancels the attachment of the IMSI in the MME by sending a CLR via the S6a interface.

### 5.3.4	Support for PDU session continuity during intersystem mobility procedures

When the UE is connected to 5GS or 5GC-N3IWF, the PGW-C+SMF may store in UDM the PGW-C+SMF FQDN for S5/S8/S2b interface, within the UE context in SMF data, when the PWG-C+SMF registers in UDM. The AMF serving 3GPP access may also notify the UDM to store the association between DNN and PGW-C+SMF FQDN which supports EPS interworking as Intersystem continuity context.

During mobility from 5GC to EPC or EPC/ePDG, or mobility from 5GC-N3IWF to EPC, or mobility from EPC to EPC/ePDG, the HSS uses the Nudm_SDM_Get service operation to retrieve the PGW-C+SMF FQDN for S5/S8/S2b interface from UDM, and subscribes to be notified using Nudm_SDM_Subscribe when the Intersystem continuity context data or the UE context in SMF data are modified. The HSS retrieves both the UE context in SMF data and the Intersystem continuity context data from UDM. The HSS checks the Intersystem continuity context first. If no PGW-C+SMF FQDN associated with the APN exists in Intersystem continuity context, the HSS selects one of the PGW-C+SMF FQDN for the APN from the UE context in SMF data based on operator's policy.

During UE registration for the scenario when ePDG is connected to SMF+PGW-C and S6b in not used, the HSS subscribers to be notified from the UDM using Nudm_SDM_Subscribe when the UE context in SMF data is modified.

Figure 5.3.4-1 shows the interaction between the HSS and the UDM in an interworking scenario when the UE attaches to the EPC or EPC/ePDG.

Figure 5.3.4-1: PDU session continuity during mobility from 5GC to EPC

0.	While the UE is connected to the 5GC, the AMF and/or SMF may store in UDM the association between DNN and PGW-C+SMF FQDN which supports EPS interworking.

If the 5GS UDR is used, the UDM updates the 5GS-UDR with the DNN and PGW-C+SMF FQDN association.

1.	The HSS receives an S6a ULR or SWx SAR request containing the IMSI of the subscriber.

The HSS reads the subscription information related to the IMSI. If the EPC-UDR is used, the HSS retrieves the subscription information from the EPS-UDR. The UE's subscription may include subscribed APNs that support interworking with 5GS.

2.	If the subscription information related to the IMSI includes subscribed APNs that support interworking with 5GS, then the HSS uses the Nudm_SDM_Get service operation to fetch the UE context in SMF data and the Intersystem continuity context data for the user, if any. The HSS includes the user's IMSI received in step 1 as the user's SUPI in this request.

3.	The UDM responds to the HSS.

The HSS checks the Intersystem continuity context first. If no PGW-C+SMF FQDN associated with an DNN exists in Intersystem continuity context, the HSS selects one of the PGW-C+SMF FQDN for the APN from the UE context in SMF data based on operator's policy.

4-5.The HSS subscribes to be notified using Nudm_SDM_Subscribe service operation when the Intersystem continuity context data or the UE context in SMF data are modified.

6.	The HSS responds to the MME with an S6a-ULA response or to the AAA with an SWx-SAA response including the PGW-C+SMF FQDN associated with subscribed APNs that support interworking with 5GS, if any. This step may happen at any time after step 3, not necessarily after steps 4-5.

Figure 5.3.4-2 shows the interaction between the HSS and the UDM when the HSS receives notification from the UDM when the data requested is modified.

Figure 5.3.4-2: Notification of PGW-C+SMF FQDN from UDM to HSS

0.	The AMF and/or SMF may add or remove or update the association between DNN and PGW-C+SMF FQDN stored in the UDM.

If the 5GS UDR is used, the UDM updates the 5GS-UDR with the DNN and PGW-C+SMF FQDN association.

1.	If the UDM finds the UE context in SMF data or the Intersystem continuity context is changed, and the active subscription from the HSS exists for the UE to be notified on the change of the UE context in SMF data or the Intersystem continuity context, then the UDM uses the Nudm_SDM_Notification service operation to notify the HSS. As part of this notification the UDM informs the HSS whether the SMF indicated that the access to 5GC was via an ePDG.

2.	The HSS responds to the UDM.

The HSS checks the change of the association between APN and PGW-C+SMF FQDN. If multiple PGW-C+SMF FQDN exists in the UE context in SMF data, the HSS selects one of the PGW-C+SMF FQDN for the APN based on operator's policy.

3.	The HSS may further trigger PDN GW Identify Notification procedure, as defined in clause 12.1.4 and clause 12.1.5 in TS 23.402.

Figure 5.3.4-3 shows the interaction between the HSS and the UDM when the HSS unsubscribes from the UDM to be notified when the data requested is modified.

Figure 5.3.4-3: Unsubscribe of PGW-C+SMF FQDN Notification from HSS to UDM

0.	The HSS checks the conditions to unsubscribe the notification of the UE context in SMF data or the Intersystem continuity context from the UDM, when the UE's 3GPP access or Non-3GPP access registration in EPC is not valid. For example, when the MME performs purge UE procedure, or the AAA performs UE deregistration towards the HSS, or the registration of the user in EPC is cancelled due to mobility from EPC to 5GC.

1.	The HSS unsubscribes to be notified using Nudm_SDM_Unsubscribe service operation towards the UDM.

2.	The UDM responds to the HSS.

When the UE is connected to EPC, the MME stores in HSS the PGW-C+SMF FQDN for S5/S8 interface via S6a reference point. When the UE is connected to EPC via an ePDG, and the S6b reference point is deployed between the PGW-C+SMF and AAA, the AAA stores in HSS the PGW-C+SMF FQDN for S5/S8 interface via SWx reference point.

NOTE:	When S6b is not deployed between the PGW-C+SMF and AAA, the registration and de-registration of the PDN GW address is performed in the UDM using the N10 interface instead. In this case, for interworking between EPC/ePDG and 5GS the UDM is not required to contact the HSS to fetch the PDN GW address.

During mobility from EPC or EPC/ePDG to 5GC, or mobility from EPC to 5GC-N3IWF, the UDM uses the Nhss_SDM_Get service operation to retrieve the PGW-C+SMF FQDN for S5/S8 interface from HSS and subscribes to be notified using Nhss_SDM_Subscribe when the data requested is modified.

Figure 5.3.4-4 shows the interaction between the HSS and the UDM in an interworking scenario when the UE moves from EPC or EPC/ePDG to 5GC.

Figure 5.3.4-4: PDU session continuity during mobility from EPC/ePDG to 5GC

0.	While the UE is connected to the EPC or EPC/ePDG, the MME or the AAA informs the HSS of the PGW-C+SMF address and associated APN.

1.	UE moves to 5G coverage, the AMF registers with the UDM using Nudm_UECM_Registration for 3GPP access.

2.	The AMF retrieves the Access and Mobility Subscription data, SMF Selection Subscription data and UE context in SMF data using Nudm_SDM_Get.

3.	If the subscription information related to the SUPI includes subscribed DNNs that support interworking with EPS, then the UDM uses the Nhss_SDM_Get service operation to fetch the PGW-C+SMF information.  The UDM includes the user's IMSI derived from SUPI.

4.	The HSS responds to the UDM.

5.	The UDM includes the PGW-C+SMF address associated with subscribed DNNs that support interworking with EPS, if any, as UE context in SMF data in Nudm_SDM_Get_Response.

6.	After a successful response is received, the AMF subscribes to be notified when the data requested is modified using Nudm_SDM_Subscribe service operation.

7.	If the UE context in SMF data is subscribed to be notified, the UDM subscribes towards the HSS (if not already subscribed, for the same UE) to be notified when the PGW-C+SMF information is modified using Nhss_SDM_Subscribe service operation.

8.	The HSS responds to the UDM.

9.	The UDM responds to the AMF.

Figure 5.3.4-5 shows the interaction between the HSS and the UDM when the UDM receives notification from the HSS when the data requested is modified.

Figure 5.3.4-5: Notification of PGW-C+SMF FQDN from HSS to UDM

0.	The MME or the AAA may add or remove or update the association between APN and PGW-C+SMF FQDN stored in the HSS.

1.	If the HSS finds the association between APN and PGW-C+SMF FQDN is changed, and the active subscription from the UDM exists for the UE to be notified on the change of the data requested, then the HSS uses the Nhss_SDM_Notification service operation to notify the UDM.

2.	The UDM responds to the HSS.

3 - 4.	If the UDM finds the association between DNN and PGW-C+SMF FQDN is changed, and the active subscription from the AMF exists for the UE to be notified on the change of the UE context in SMF data, then the UDM notifies the AMF using the Nudm_SDM_Notification service operation.

Figure 5.3.4-6 shows the interaction between the HSS and the UDM when the UDM unsubscribes from the HSS to be notified when the data requested is modified.

Figure 5.3.4-6: Unsubscribe of PGW-C+SMF FQDN Notification from UDM to HSS

1 - 2.	The AMF unsubscribes to be notified using Nudm_SDM_Unsubscribe service operation.

3.	If the UE context in SMF data is unsubscribed, and no other AMF has subscribed to the same data (UE context in SMF) for the same UE, the UDM unsubscribes towards the HSS to be notified when the PGW-C+SMF information is modified using Nhss_SDM_Unsubscribe service operation.

4.	The HSS responds to the UDM.

## 5.4	Scenarios of Interworking with IMS

### 5.4.1	T-ADS

Figure 5.4.1-1 shows the scenario where the HSS receives a T-ADS request from the AS  for a subscriber who has a 5GC subscription.

Figure 5.4.1-1: T-ADS request for 5G subscriber

1.	The HSS receives a T-ADS request from the AS. The request includes the user's IMPU or MSISDN and optionally the user's IMPI.

NOTE:	An SBA capable IMS-AS makes use of Nhss_ImsSDM service to interact with the HSS for this operation as defined in 3GPP TS 23.228 [7]. Otherwise the IMS-AS makes use of Diameter Sh-UDR/UDA command.

2.	If the EPS-UDR is used, the HSS reads subscription data from the EPS-UDR. The HSS, retrieves the user's IMSI associated with the user's identifiers received in step 1. The subscription data may contain an indication that the user has a 5GC subscription.

3.	The HSS performs T-ADS info retrieval in the EPC. This may include contacting the MME and/or SGSN.

4.	If the HSS detects in step 2 that the user has a 5GC subscription and unless the user is known not to be registered in 5GC (e.g. in case the user is registered in EPC and the HPLMN is working in single registration mode with N26), the HSS uses the Nudm_MT_ProvideDomainSelection service operation to retrieve T-ADS information from the UDM. The HSS includes the user's IMSI associated with the user's identifiers received in step 1 as the SUPI in this request.

5.	The UDM reads data from the 5GS-UDR to see whether the registered AMF(s) support IMS voice over PS.

6-7.	If support/non-support of IMS voice is not homogeneous for the User in the AMF for 3GPP access, the UDM retrieves domain selection information from the AMF.

8.	The UDM provides the 5G domain selection info to the HSS.

9.	The HSS combines the 5G domain selection info with the EPS domain selection info from step 3 and sends the result to the AS.

### 5.4.2	P-CSCF Restoration

Figure 5.4.2-1 shows the scenario where the HSS receives a P-CSCF restoration indication from the S-CSCF for a subscriber who has a 5GC subscription.

Figure 5.4.2-1: P-CSCF Restoration for 5G subscriber

1.	The HSS receives a P-CSCF Restoration indication from the S-CSCF. The request includes the user's IMPU and IMPI.

NOTE:	An SBA capable S-CSCF makes use of Nhss_ImsUECM service to interact with the HSS for this operation as defined in 3GPP TS 23.228 [7]. Otherwise the S-CSCF makes use of Diameter Cx-SAR/SAA command.

2.	If the EPS-UDR is used, the HSS reads subscription data from the EPS-UDR. The HSS, retrieves the user's IMSI associated with the user's identifiers received in step 1. The subscription data may contain an indication that the user has a 5GC subscription.

3-4.	The HSS acknowledges receipt of the request and performs P-CSCF restoration in the EPC. For details see 3GPP TS 23.380 [4] clause 5.4.2.1.

5.	If the HSS detects in step 2 that the user has a 5GC subscription and unless the user is known not to be registered in 5GC (e.g. in case the user is registered in EPC and the HPLMN is working in single registration mode with N26), the HSS uses the Nudm_UECM_P-CscfRestorationTrigger service operation to trigger P-CSCF restoration in 5GC. See also 3GPP TS 23.380 [4] clause 5.8.4. The HSS includes the user's IMSI associated with the user's identifiers received in step 1 as the SUPI in this request

6.	The UDM reads data from the 5GS-UDR to see which NFs have subscribed to P-CSCF Restoration Notifications (implicitly during registration).

7.	The UDM acknowledges receipt of Nudm_UECM_PCscfRestorationTrigger to the HSS.

8-9.	The UDM notifies all subscribed NFs (AMF, SMF).

### 5.4.3	Network Provided Location Information Request

Figure 5.4.3-1 shows the scenario where the HSS receives a Location Information retrieval request from the IMS-AS for a subscriber who has a 5GC subscription.

Figure 5.4.3-1: Location Information retrieval request for 5G subscriber

1.	The HSS receives a Location Information retrieval request (requested nodes includes AMF) from the IMS-AS.

NOTE:	An SBA capable IMS-AS makes use of Nhss_ImsSDM service to interact with the HSS for this operation as defined in 3GPP TS 23.228 [7]. Otherwise the IMS-AS makes use of Diameter Sh-UDR/UDA command.

2.	The HSS reads subscription data from the EPS-UDR.

3.	The HSS performs Location Information Retrieval from other nodes if so requested. This may include contacting the MME and/or SGSN.

4.	If the HSS detects in step 2 that the user has a 5GC subscription and unless the user is known not to be registered in 5GC, the HSS uses the Nudm_MT_ProvideLocationInfo service operation to retrieve 5GS Location Information. The service request includes indicators for Current Location, Serving Node Indication, Local Time Zone Indication, and RAT-Type requested as has been received in step 1.

5.	The UDM reads data from the 5GS-UDR to get the AMF and SMSF for 3GPP Access Registration Information. If the Serving Node Indication was received in step 4, steps 6-7 shall be skipped.

6-7.	The UDM retrieves location information from the AMF (for 3GPP access) by means of the Namf_Location_ProvideLocationInfo service operation .

8.	The UDM provides the 5GS location information info to the HSS.

9.	The HSS provides the requested location information to the IMS-AS.

### 5.4.4	User State Retrieval

Figure 5.4.4-1 shows the scenario where the HSS receives a PS User State retrieval request from the IMS-AS for a subscriber who has a 5GC subscription.

Figure 5.4.4-1: PS User State retrieval request for 5G subscriber

1.	The HSS receives a PS User State retrieval request (requested nodes includes AMF) from the IMS-AS.

NOTE 1:	An SBA capable IMS-AS makes use of Nhss_ImsSDM service to interact with the HSS for this operation as defined in 3GPP TS 23.228 [7]. Otherwise the IMS-AS makes use of Diameter Sh-UDR/UDA command.

2.	The HSS reads subscription data from the EPS-UDR.

3.	The HSS performs PS User State Retrieval from other nodes if so requested. This may include contacting the MME and/or SGSN.

4.	If the request includes AMF as requested node and if the HSS detects in step 2 that the user has a 5GC subscription and unless the user is known not to be registered in 5GC, the HSS uses the Nudm_MT_ProvideUserState service operation to retrieve the 5GS User State. Otherwise, continue with step 9.

5.	The UDM reads data from the 5GS-UDR to get the AMF for 3GPP Access Registration Information.

6-7.	The UDM retrieves the user state from the AMF (for 3GPP access) by means of the Namf_EventExposure Subscribe service operation (one time immediate report requested).

8.	The UDM provides the 5GS PS User State to the HSS.

9.	The HSS provides the requested user state to the IMS-AS.

NOTE 2:	If the IMS-AS supports 5G SBI it may make use of the Nhss-ims service instead of Sh.

### 5.4.5	UE Reachability

Figure 5.4.5-1 shows the scenario where the HSS receives a Subscription to notification request for UE-reachability from the IMS-AS  for a subscriber who has a 5GC subscription.

Figure 5.4.5-1: Subscription to UE reachability for 5G subscriber

1.	The HSS receives a Subscribe request for UE reachability for IP from the IMS-AS.

NOTE:	An SBA capable IMS-AS makes use of Nhss_ImsSDM service to interact with the HSS for this operation as defined in 3GPP TS 23.228 [7]. Otherwise the IMS-AS makes use of Diameter Sh-SNR/SNA command.

2.	The HSS reads subscription data from the EPS-UDR.

3.	The HSS sets the UE Reachability flags for EPC and if the UE is registered in EPC contacts the registered MME and SGSN to get notified when the UE becomes reachable.

4.	If the HSS detects in step 2 that the user has a 5GC subscription, the HSS uses the Nudm_EventExposure_Subscribe service operation (one time immediate report requested) to get notified when the UDM detects UE reachability for IP. Otherwise, continue with step 10.

5.	The UDM sets the URRP-AMF flag in the 5GS-UDR and reads data from the 5GS-UDR to get the AMF for 3GPP Access Registration Information and the AMF for non-3GPP Access Registration Information if any.

6-7.	[Conditional] If an AMF is registered in UDM for the target UE and the UDM has not already subscribed in AMF due to a previous subscription from a different NF, the UDM subscribes to UE reachability notifications at the registered AMFs by means of the Namf_EventExposure_Subscribe service operation (see 3GPP TS 23.502 [5]).

8.	The UDM updates the 5GS-UDR with the EE-Subscription for the HSS.

9.	The UDM acknowledges the EE Subscription to the HSS.

10.	The HSS acknowledges the Sh subscription to the IMS-AS.

11.	The HSS updates the EPS-UDR with the Sh subscription for the IMS-AS.

Figure 5.4.5-2 shows the scenario where the UDM detects UE reachability for IP and notifies the HSS that has previously subscribed.

Figure 5.4.5-2: UE reachability notification for 5G subscriber

1.	The UDM receives a Notification or Registration from the AMF.

2.	The UDM reads subscription data from the 5GS-UDR.

3.	The UDM acknowledges step 1 towards the AMF. If an old AMF is registered in the UDM, the UDM sends a Nudm_UECM_DeregistrationNotification service operation to the old AMF.

4.	The UDM notifies the HSS (and any other NF that has subscribed) about the reachability of the UE.

5.	The HSS reads data from the EPS-UDR to see whether an IMS-AS has subscribed do reachability notification.

6.	The HSS acknowledges step 4.

7.	The UDM updates the 5GS-UDR to delete the EE-Subscription(s).

8.	The HSS notifies the IMS-AS about UE reachability for IP.

NOTE:	An SBA capable IMS-AS receives the notification from HSS using the Nhss_ImsSDM service as defined in 3GPP TS 23.228 [7]. Otherwise the IMS-AS receives the notification via a Diameter Sh-PNR/PNA command.

9.	The IMS-AS acknowledges step 8.

10.	The HSS updates the EPS-UDR to delete the IMS-AS's subscription.

### 5.4.6	IMEI Retrieval

A pre-requisite for the retrieval of the IMEI when requested by an IMS-AS is that the IMEI for a given UE stored in HSS and UDM is always synchronized. This is, when the HSS detects that the IMEI for a UE changes (e.g. during an Update Location in EPS), the HSS informs the UDM about the IMEI change which stores the new IMEI accordingly. Similarly, when UDM detects that the IMEI changes, the UDM informs the HSS about the IMEI change which stores the new IMEI accordingly.

This allows that the retrieval of the IMEI for a given UE requested by an IMS-AS from HSS via Sh can be executed locally by HSS without the need of additional interworking between the HSS and the UDM

Figure 5.4.6-1 shows the scenario where the HSS receives an IMEI retrieval request from the IMS-AS for a subscriber who has a 5GC subscription.

Figure 5.4.6-1: IMEI retrieval request for 5G subscriber

Steps 1 to 4 are executed when the HSS detects a change in the IMEI for a given UE (IMSI).

1.	The HSS receives a request from the MME including an IMEI for the UE (e.g. Update Location Request or Notify Request).

2.	The HSS detects that the IMEI received in the request is different from the previously stored in the EPS-UDR. The HSS stores the new IMEI in the EPS-UDR.

3.	The HSS informs the UDM about the new PEI (IMEI) using the Nudm_UECM_Update service operation.

4.	The UDM stores the new PEI for the UE.

Steps 5 to 8 are executed when the UDM detects a change in the PEI for a given UE (SUPI).

5.	The UDM receives a request from the AMF including a PEI for the UE (e.g. Nudm_UECM_Registration/Update).

6.	The UDM detects that the PEI received in the request is different from the previously stored in the 5GS-UDR. The UDM stores the new IMEI in the 5GS-UDR.

7.	The UDM informs the HSS about the new IMEI using the Nhss_UECM_Update service operation.

8.	The HSS stores the new IMEI for the UE.

Steps 9 to 11 are executed when the HSS receives an IMEI retrieval request from the IMS-AS for a subscriber who has a 5GC subscription.

9.	The HSS receives a request from IMS-AS to retrieve the IMEI for a UE.

10.	The HSS reads the IMEI stored in the EPS-UDR.

11.	The HSS replies to the IMS-AS with the users IMEI. Since the IMEI for the UE has been sinchronized between HSS and UDM at every IMEI/PEI change event as in steps 1 to 4 or 5 to 8, the HSS can reply to the IMS-AS without any additional interworking with the UDM.

### 5.4.7	SRVCC: IMS AS obtaining SRVCC data

Figure 5.4.7-1 shows the scenario where the HSS communicates with the IMS-AS in support of (5G) SRVCC for a subscriber who has a 5GC subscription. An operator shall be able to control whether the interaction between HSS and UDM is performed based on local policy.

Figure 5.4.7-1: IMS AS obtaining data related to (5G) SRVCC and updating STN-SR

1.	As described in 3GPP TS 23.237 [13], the IMS-AS sends Sh-Pull message to the UDM/HSS in order to know the SRVCC capability indicated by the UE in the EPS, and to retrieve the STN-SR stored in the UDM/HSS.

2.	The HSS reads STN-SR and UE SRVCC Capability data from the EPS-UDR. The HSS reads the C-MSISDN bound to the IMS Private User Identity.

3.	The HSS, based on operator policy, may query UDM to retrieve SRVCC parameters. The HSS uses the Nudm_MT_Provide5GSRVCCInfo service operation to retrieve the STN-SR and the UE SRVCC Capability indicated by the UE in the 5GS (see 3GPP TS 23.008 [14]).

4.	The UDM reads the STN-SR and this UE SRVCC Capability data from the 5GS-UDR.

5.	The UDM provides the C-MSISDN, STN-SR and UE SRVCC Capability data to the HSS.

6.	The HSS provides a single C-MSISDN, a single STN-SR and UE SRVCC Capability data to the IMS-AS, as follows:

-	if the C-MSISDN from the HSS and the C-MSISDN from the UDM are empty, the HSS indicates an empty C-MSISDN to the SCC AS. The HSS indicates a non-empty C-MSISDN, otherwise;

-	if the STN-SR from the HSS or the STN-SR from the UDM is empty, it indicates the user is not subscribed to (5G) SRVCC. If the HSS and the UDM indicate the user is not subscribed to (5G) SRVCC, the HSS indicates an empty STN-SR to the SCC AS. The HSS indicates a non-empty STN-SR, otherwise.

Editor's note: procedures for obtaining the UE SRVCC Capability/ies and returning them to SCC-AS are FFS.

### 5.4.8	SRVCC: IMS AS obtaining SRVCC data – HSS using Nudr

When the HSS receives an S6a-AIR from the MME, it may check (by means of an EPC-UDR query) whether the subscriber has a 5G subscription. If so, the HSS can use of the Nudr_DM_Query Get service operation to retrieve the 5G SRVCC subscription data from the 5GS UDR.

Figure 5.4.8-1 shows the scenario where the HSS communicates with the IMS-AS in support of (5G) SRVCC for a subscriber who has a 5GC subscription. An operator shall be able to control whether the interaction between HSS and 5GS-UDR is performed based on local policy.

Figure 5.4.8-1: IMS AS obtaining data related to (5G) SRVCC – HSS using the Nudr

1.	As described in 3GPP TS 23.237 [13], the IMS-AS sends Sh-Pull message to the UDM/HSS in order to know the SRVCC capability indicated by the UE in the EPS, and to retrieve the STN-SR stored in the UDM/HSS.

2.	The HSS reads STN-SR and this UE SRVCC Capability data from the EPS-UDR. The HSS reads the C-MSISDN bound to the IMS Private User Identity.

3.	The HSS, based on operator policy, may query UDM to retrieve SRVCC parameters. The HSS uses the Nudr_DM_Query service operation to retrieve the STN-SR and the UE SRVCC Capability indicated by the UE in the 5GS (see 3GPP TS 23.008 [14]) from the 5GS-UDR.

4.	The HSS provides a single C-MSISDN, a single STN-SR and UE SRVCC Capability data to the IMS-AS, as follows:

-	if the C-MSISDN from the HSS and the C-MSISDN from the UDM are empty, the HSS indicates an empty C-MSISDN to the SCC AS. The HSS indicates a non-empty C-MSISDN, otherwise;

-	if the STN-SR from the HSS or the STN-SR from the UDM is empty, it indicates the user is not subscribed to (5G) SRVCC. If the HSS and the UDM indicate the user is not subscribed to (5G) SRVCC, the HSS indicates an empty STN-SR to the SCC AS. The HSS indicates a non-empty STN-SR, otherwise.

Editor's note: procedures for obtaining the UE SRVCC Capability/ies and returning them to SCC-AS are FFS.

### 5.4.9	SRVCC: IMS AS updating STN-SR

Figure 5.4.9-1 shows the scenario where the IMS-AS updates the STN-SR at the HSS. An operator shall be able to control whether the interaction between HSS and UDM is performed based on local policy.

Figure 5.4.9-1: Updating STN-SR

1.	As described in 3GPP TS 23.237 [13], the IMS-AS can use Sh-Update to provide the STN-SR received from the ATCF to the UDM/HSS in order to replace the STN-SR at the UDM/HSS.

2.	While this updated STN-SR is a transient value and subject to the user being subscribed to SRVCC, the HSS can update the STN-SR at the EPS-UDR. If the user is subscribed to SRVCC, the HSS updates the STN-SR at the EPS-UDR and at the MME/SGSN (not shown).

3.	The HSS, based on operator policy, may update UDM: the HSS updates the STN-SR at the UDM.

4.	While this updated STN-SR is a transient value, the UDM can update the STN-SR at the 5GS-UDR using Nudr_DM_Update. The UDM updates the STN-SR at the AMF using Nudm_SDM_Notification service operation (not shown).

5.	The UDM responds to the request to update the STN-SR.

6.	The HSS responds by sending Sh-Update Response message to the IMS AS.

## 5.5	SMS Support

### 5.5.1	General

Potential MT-SMS target nodes in the EPS are the MME/MSC and the SGSN registered at the HSS, while in 5GS MT-SMS target nodes are the 3GPP-SMSF and the Non3GPP-SMSF registered at the UDM. For the role of SMS-Router and IP-SM-GW see 3GPP TS 23.040 [12].

### 5.5.2	MT-SMS Routing Information Retrieval

Figure 5.5.2-1 shows the interaction when the SMS-GMSC retrieves routing information from the HSS for MT-SMS delivery.

Figure 5.5.2-1: SMS Routing Info Retrieval

1. The HSS receives a request for routing information from the SMS-GMSC via MAP or S6c.

2. The HSS queries the EPS-UDR via Ud to read the registered MME/MSC, the registered SGSN, the UE-not-reachable flags for MME/MSC, SGSN, 3GppSMSF and Non3GppSMSF and the SMSF Registration Notification flag.

3. If the UE-not-reachable flags for 3GppSMSF, Non3GppSMSF and SMSF Registration Notification flag are not set and unless the user is known not to be registered in 5GC, the HSS retrieves the registered SMSF addresses (if any) from the UDM.

4-5. The UDM retrieves the requested information from the 5GS-UDR.

6. The UDM forwards the retrieved addresses to the HSS if any.

7. The HSS returns the relevant MT-SMS target node addresses registered in HSS and/or UDM to the SMS-GMSC and the procedure is terminated.

Otherwise, if there is no MT-SMS target node address registered in HSS nor in UDM, a negative response (Absent Subscriber SM) is sent to the SMS-GMSC and the procedure continues with steps 8 to 11.

8. The HSS includes the SMSC address to the Message Waiting Data (MWD) stored in the EPS-UDR and informs the SMSC as defined in TS 23.040 [12]. The relevant UE-not-reachable flags and the SMSF Registration Notification flag are set in the EPS-UDR.

9. The HSS subscribes in UDM to be notified when the UE registers in the 5GC for SMS service (i.e. when an SMSF is registered in UDM) by using the Nudm_EE_Subscribe service operation (SUPI, SMSF Registration Notification event) as defined in 3GPP TS 23.502 [5].

NOTE:	In this case, the HSS subscribes to SMSF Registration Notification event in UDM instead to the UE Reachability Notification as even if the UE is reachable in an AMF, the UE will not be ready for SMS within the 5GC until an SMSF is registered for the UE in UDM.

10. The UDM stores the SMSF Registration Notification flag in the 5G-UDR.

11. The UDM acknowledges the subscription to the HSS.

### 5.5.3	MT-SMS Delivery Failure

Figure 5.5.3-1 shows the interaction when the SMS-GMSC sends Report-SM-Delivery-Status to the HSS.

Figure 5.5.3-1: MT-SMS Delivery Failure

1. The HSS receives a Report-SM-Delivery-Status from the SMS-GMSC indicating the MT-SMS target nodes at which MT-SMS delivery was unsuccessful.

2-3. The HSS reads and updates the Message Waiting Data stored in the EPS-UDR.

4. The HSS acknowledges the receipt of the delivery status to the SMS-GMSC.

5. If during step 3 one of the UE-not-reachable flags for 3GppSMSF and Non3GppSMSF were modified from false to true, the HSS subscribes to notification on UE-Reachability for SMS at the UDM, using the Nudm_EE_Subscribe service operation as defined in 3GPP TS 23.502 [5].

6-7. The UDM queries the 5GS-UDR to see whether UE-Reachability has already been subscribed at the registered AMF(s) (i.e. whether URRP-AMF flags are set).

8-9. If not already subscribed, the UDM subscribes to UE-Reachability notification at the AMF(s) using the Namf_EE service.

10-11. The UDM stores the received EE-Subscription in the 5GS-UDR and if steps 8-9 were performed, the UDM sets the relevant URRP-AMF flags in the 5GS-UDR.

12. The UDM acknowledges the subscription to the HSS.

### 5.5.4	SMS Alerting

Figure 5.5.4-1 shows the interaction when the UE becomes available.

Figure 5.5.4-1: SMS Alerting

1. The UDM receives a Notification  from the AMF or an SMSF Registration.

2-3. The UDM queries the 5GS-UDR to see whether any NF (e.g. the HSS) has subscribed to notifications on UE-reachability for SMS or SMSF Registration events.

4. The UDM acknowledges the message received in step 1.

5. If the HSS has subscribed to UE reachability for SMS notification or an SMSF Registration, the UDM notifies the HSS.

6-7. The UDM updates (clears) the relevant URRP-AMF and the SMSF Registration Notification flags and deletes the (one-time) EE-subscriptions of UE reachability for SMS or SMSF Registration Notification event in the 5GS-UDR.

8. The HSS reads Message Waiting Data from the EPS-UDR.

9-10. The HSS sends Alert-SC to all SMS-GMSCs stored in the MWD.

11. The HSS removes the SMS-GMSCs from the MWD stored in the EPS-UDR and clears the relevant UE-not-reachable flags or the SMSF Registration Notification flag from the EPS-UDR.

12. The HSS acknowledges receipt of the Notification to the UDM.

Editor's Note:	It is FFS to confirm that the details of this procedure are aligned with the contents of CR 1715 to TS 23.502.

### 5.5.5	MT-SMS Routing Information Retrieval Over Nudr

Figure 5.5.5-1 shows the interaction when the SMS-GMSC retrieves routing information from the HSS for MT-SMS delivery when the HSS uses the Nudr SBI.

Figure 5.5.5-1: SMS Routing Info Retrieval over Nudr

1. The HSS receives a request for routing information from the SMS-GMSC via MAP or S6c.

2. The HSS queries the EPS-UDR via Ud to read the registered MME/MSC, the registered SGSN and the UE-not-reachable flags for MME/MSC, SGSN, 3GppSMSF and Non3GppSMSF and the SMSF Registration Notification flag.

3-4. If the UE-not-reachable flags for 3GppSMSF and Non3GppSMSF and SMSF Registration Notification flag are not set and unless the user is known not to be registered in 5GC, the HSS retrieves the registered SMSF addresses (if any) from the 5GS-UDR.

5. The HSS returns the relevant MT-SMS target node addresses registered in HSS and/or UDM to the SMS-GMSC and the procedure is terminated.

Otherwise, if there is no MT-SMS target node address registered in HSS nor in UDM, a negative response (Absent Subscriber SM) is sent to the SMS-GMSC and the procedure continues with steps 8 to 11 in figure 5.5.2-1.

# 6	Network Function Service procedures

## 6.1	HSS Services

### 6.1.1	General

The following table illustrates the HSS Services.

Table 6.1.1-1: NF services provided by HSS

| NF service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| UE Authentication | Get | Request/Response | UDM |
| UECM | MMEDeregistration | Request/Response | UDM |
|  | Update | Request/Response | UDM |
| SDM | Get | Request/Response | UDM |
|  | Subscribe | Subscribe/Notify | UDM |
|  | Unsubscribe |  |  |
|  | Notify |  |  |

### 6.1.2	Nhss_UEAuthentication service

#### 6.1.2.1	Nhss_UEAuthentication_Get service operation

Service operation name: Nhss_UEAuthentication_Get

Description: Requester NF gets the authentication vector from HSS. For AKA based authentication, this operation can be also used to recover from synchronization failure situations.

Inputs, Required: IMSI, authentication method, serving network name.

Inputs, Optional: Synchronization Failure indication and related information (i.e. RAND/AUTS).

Outputs, Required: Authentication vector.

Outputs, Optional: None.

### 6.1.3	Nhss_UECM service

#### 6.1.3.1	Nhss_UECM_MMEDeregistration service operation

Service operation name: Nhss_UECM_MMEDeregistration.

Description: The NF consumer requests the HSS to initiate cancel location to the MME serving the UE, if any.

Inputs, Required: IMSI, deregistration reason.

Inputs, Optional: None.

Outputs, Required: Result Indication.

Outputs, Optional: None.

#### 6.1.3.2	Nhss_UECM_Update service operation

Service operation name: Nhss_UECM_Update.

Description: The NF consumer informs the HSS about an update in the UE Context (e.g. a change in the IMEI).

Inputs, Required: IMSI.

Inputs, Optional: IMEI.

Outputs, Required: Result Indication.

Outputs, Optional: None.

### 6.1.4	Nhss_SDM service

#### 6.1.4.1	Nhss_SDM_Get service operation

Service operation name: Nhss_SDM_Get

Description: The consumer NF gets the subscriber data indicated by the requested data type from HSS. In this release, only the PGW-C+SMF FQDN for S5/S8/S2b interface information is supported as requested data type.

Inputs, Required: IMSI, requested data.

Inputs, Optional: None.

Outputs, Required: The consumer NF gets the requested subscription data

Outputs, Optional: None.

#### 6.1.4.2	Nhss_SDM_Notification service operation

Service or service operation name: Nhss_SDM_Notification

Description: The HSS notifies NF consumer of the updates of the subscriber data indicated by the "subscription data Type" input. In this release, only the PGW-C+SMF FQDN for S5/S8/S2b interface information is supported as subscription data type.

Inputs, Required: IMSI, Subscription data type(s).

Inputs, Optional: None.

Outputs, Required: Result Indication.

The HSS invokes this service operation under the following cases:

-	When the PGW-C+SMF FQDN for S5/S8/S2b interface information is updated at the HSS, the updated information is notified to the NF consumer that has subscribed for the specific subscription data type to be notified.

Outputs, Optional: None.

#### 6.1.4.3	Nhss_SDM_Subscribe service operation

Service operation name: Nhss_SDM_Subscribe

Description: The NF consumer subscribes for updates to the subscriber data indicated by the 'subscription data type' input. The HSS shall check the requested consumer is authorized to subscribe to requested updates. In this release, only the PGW-C+SMF FQDN for S5/S8/S2b interface information is supported as subscription data type.

Inputs, Required: IMSI, Subscription data type(s).

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

#### 6.1.4.4	Nhss_SDM_Unsubscribe service operation

Service operation name: Nhss_SDM_Unsubscribe

Description: The NF consumer unsubscribes from updates to the subscriber data indicated by the 'subscription data type' input. In this release, only the PGW-C+SMF FQDN for S5/S8 interface information is supported as subscription data type.

Inputs, Required: IMSI, Subscription data type(s).

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

## 6.2	UDM Services

### 6.2.1	General

The following table illustrates the UDM Services.

Table 6.2.1-1: NF services provided by UDM

| NF service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| UECM | P-CscfRestorationTrigger | Request/Response | HSS |
|  | Get | Request/Response | HSS |
|  | AMFDeregistration | Request/Response | HSS |
|  | Update | Request/Response | HSS |
| MT | ProvideDomainSelectionInfo | Request/Response | HSS |
|  | ProvideUserState | Request/Response | HSS |
|  | ProvideLocationInfo | Request/Response | HSS |
|  | Provide5GSRVCCInfo | Request/Response | HSS |
| EventExposure | Subscribe | Subscribe/Notify | HSS |
|  | Notify |  |  |
| UEAU | GetHssAv | Request/Response | HSS |
| SDM | Get | Request/Response | HSS |
|  | Subscribe | Subscribe/Notify | HSS |
|  | Unsubscribe |  |  |
|  | Notify |  |  |
| PP | Update | Request/Response | HSS |

### 6.2.2	Nudm_UECM service operation

#### 6.2.2.1	Nudm_UECM_P-CscfRestorationTrigger service operation

Service operation name: Nudm_UECM_P-CscfRestorationTrigger

Description: Requester NF (HSS) triggers the UDM to notify all NFs (AMF, SMF) that have previously subscribed to receive notifications for P-CSCF restoration.

Inputs, Required: SUPI.

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

#### 6.2.2.2	Nudm_UECM_Get service operation

See 3GPP TS 23.502 [5] clause 5.2.3.2.4.

#### 6.2.2.3	Nudm_UECM_AMFDeregistration service operation

Service operation name: Nudm_UECM_AMFDeregistration.

Description: The NF consumer requests the UDM to initiate deregistration of the AMF serving the UE, if any.

Inputs, Required: SUPI, deregistration reason.

Inputs, Optional: None.

Outputs, Required: Result Indication.

Outputs, Optional: None.

#### 6.2.2.4	Nudm_UECM_Update service operation

See 3GPP TS 23.502 [5] clause 5.2.3.2.5.

### 6.2.3	Nudm_MT Service

#### 6.2.3.1	Nudm_MT_ProvideDomainSelectionInfo Service

Service operation name: Nudm_MT_ProvideDomainSelectionInfo

Description: Provides the UE information for terminating domain selection of IMS voice in 5G to the consumer NF (HSS).

Inputs, Required: SUPI.

Inputs, Optional: None.

Outputs, Required: Success/Failure indication.

Outputs, Optional: Indication of supporting IMS voice over PS Session or not, Time stamp of the last radio contact with the UE, Current RAT type.

#### 6.2.3.2	Nudm_MT_ProvideUserState service operation

Service operation name: Nudm_MT_ProvideUserState

Description: Provides the UE's 5GS PS User State to the consumer NF (HSS).

Inputs, Required: SUPI.

Inputs, Optional: None.

Outputs, Required: Success/Failure indication.

Outputs, Optional: 5GS User State.

#### 6.2.3.3	Nudm_MT_ProvideLocationInfo service operation

Service operation name: Nudm_MT_ProvideLocationInfo

Description: Provides the UE's 5GS location information to the consumer NF (HSS).

Inputs, Required: SUPI, Current Location Indicator, Serving Node Indication, Local Time Zone Indication, and RAT-Type requested Indicator.

Inputs, Optional: None.

Outputs, Required: Success/Failure indication.

Outputs, Optional: 5GS location information.

#### 6.2.3.4	Nudm_MT_Provide5GSRVCCInfo Operation

Service operation name: Nudm_MT_ProvideSRVCCInfo

Description: Provides the UE PS to CS SRVCC capability for the UE to the consumer NF (HSS).

Inputs, Required: SUPI.

Inputs, Optional: None.

Outputs, Required: Success/Failure indication.

Outputs, Optional: Indication of the UE supporting SRVCC capability or not, If the 5GS supports 5G-SRVCC, an STN-SR and a C-MSIDSN.

### 6.2.4	Nudm_EE Service

#### 6.2.4.1	Nudm_EventExposure_Subscribe service operation

See 3GPP TS 23.502 [5] clause 5.2.3.5.2.

When used by the HSS in MT-SMS delivery failure scenarios (see clause 4.6.3) the subscribe request shall be a one-time subscription for UE-reachability for SMS and shall indicate whether it is applicable to 3GPP access, Non3GPP access or both.

#### 6.2.4.2	Nudm_EventExposure_Notify service operation

See 3GPP TS 23.502 [5] clause 5.2.3.5.4.

When used by the HSS in SMS Alerting scenarios (see clause 4.6.4) the notify request shall indicate whether it is applicable to 3GPP access or Non3GPP access.

### 6.2.5	Nudm_UEAuthentication service

#### 6.2.5.1	Nudm_UEAuthentication_GetHssAv service operation

Service operation name: Nudm_UEAuthentication_GetHssAv

Description: Requester NF gets the authentication vector from UDM. For AKA based authentication, this operation can be also used to recover from synchronization failure situations.

Inputs, Required: SUPI, authentication method (e.g. EAP-AKA', EPS-AKA, IMS-AKA, GBA-AKA), serving network name.

Inputs, Optional: Synchronization Failure indication and related information (i.e. RAND/AUTS).

Outputs, Required: Authentication vector.

Outputs, Optional: None.

### 6.2.6	Nudm_SDM service operations

#### 6.2.6.1	Nudm_SDM_Get service operation

See 3GPP TS 23.502 [5] clause 5.2.3.3.2.

#### 6.2.6.2	Nudm_SDM_Subscribe service operation

See 3GPP TS 23.502 [5] clause 5.2.3.3.4.

#### 6.2.6.3	Nudm_SDM_Unsubscribe service operation

See 3GPP TS 23.502 [5] clause 5.2.3.3.5.

#### 6.2.6.4	Nudm_SDM_Notify service operation

See 3GPP TS 23.502 [5] clause 5.2.3.3.3.

### 6.2.7	Nudm_PP service operations

#### 6.2.7.1	Nudm_PP_Update service operation

Service operation name: Update

Description: Removes, provides or modifies the STN-SR, CMSISDN, if the 5GS support 5G-SRVCC.

Inputs, Required: None.

Inputs, Optional: SUPI, STN-SR, CMSISDN.

Outputs, Required: Success/Failure indication.

Outputs, Optional: None.

## 6.3	UDR Services

The UDM and optionally the HSS shall make use of the Nudr_DataRepository Query service operations specified in 3GPP TS 23.502 [5] to retrieve Authentication Subscription Data. If the subscribed authentication method is 5G_AKA or EAP_AKA_PRIME, the AuthenticationSubscriptionData shall include an indicator indicating whether for the identified subscriber authentication vector generation is to be performed by the HSS.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-04 | CT4#90 | C4-191319 |  |  |  | Skeleton | 0.1.0 |
| 2019-04 | CT4#90 | C4-191320 |  |  |  | Scope | 0.2.0 |
| 2019-04 | CT4#90 | C4-191321 |  |  |  | Reference Architecture | 0.2.0 |
| 2019-05 | CT4#91 | C4-192135 |  |  |  | Removal of Editor's Note | 0.3.0 |
| 2019-05 | CT4#91 | C4-192402 |  |  |  | P-CSCF Restoration | 0.3.0 |
| 2019-05 | CT4#91 | C4-192403 |  |  |  | T-ADS | 0.3.0 |
| 2019-05 | CT4#91 | C4-192405 |  |  |  | SMS | 0.3.0 |
| 2019-05 | CT4#91 | C4-192462 |  |  |  | Authentication | 0.3.0 |
| 2019-05 | CT4#91 | C4-192098 |  |  |  | Authentication – HSS using Nudr | 0.3.0 |
| 2019-09 | CT4#93 | C4-193134 |  |  |  | System Architecture | 0.4.0 |
| 2019-09 | CT4#93 | C4-193137 |  |  |  | IMS procedures | 0.4.0 |
| 2019-09 | CT4#93 | C4-193829 |  |  |  | Mobility procedures | 0.4.0 |
| 2019-09 | CT4#93 | C4-193556 |  |  |  | Authentication vector generation in UDM | 0.4.0 |
| 2019-09 | CT4#93 | C4-193297 |  |  |  | MT-SMS Routing Information Retrieval Over Nudr | 0.4.0 |
| 2019-09 | CT4#93 | C4-193617 |  |  |  | Location Info Retrieval | 0.4.0 |
| 2019-09 | CT4#93 | C4-193618 |  |  |  | UE reachability | 0.4.0 |
| 2019-09 | CT4#93 | C4-193619 |  |  |  | User State Info Retrieval | 0.4.0 |
| 2019-10 | CT4#94 | C4-194347 |  |  |  | ProvideLocationInformation | 0.5.0 |
| 2019-10 | CT4#94 | C4-194348 |  |  |  | Document cleanup | 0.5.0 |
| 2019-10 | CT4#94 | C4-194350 |  |  |  | IMS Use Cases Correction | 0.5.0 |
| 2019-10 | CT4#94 | C4-194252 |  |  |  | N26 Editor's Note Removal | 0.5.0 |
| 2019-11 | CT4#95 | C4-195416 |  |  |  | HSS to 5GS-UDR reference point | 0.6.0 |
| 2019-11 | CT4#95 | C4-195020 |  |  |  | P-CSCF restoration | 0.6.0 |
| 2019-11 | CT4#95 | C4-195420 |  |  |  | Clarification on Mobility, IMS and SMS | 0.6.0 |
| 2019-11 | CT4#95 | C4-195137 |  |  |  | P-CSCF Restoration Procedure | 0.6.0 |
| 2019-11 | CT4#95 | C4-195138 |  |  |  | Nudm_UEAuthentication service operations | 0.6.0 |
| 2019-11 | CT4#95 | C4-195428 |  |  |  | Nudm_SDM Service | 0.6.0 |
| 2019-11 | CT4#95 | C4-195429 |  |  |  | SMS Procedures | 0.6.0 |
| 2019-12 | CT#86 | CP-193066 |  |  |  | TS presented for information and approval | 1.0.0 |
| 2019-12 | CT#86 | CP-193296 |  |  |  | A title corrected | 1.0.1 |
| 2019-12 | CT#86 |  |  |  |  | TS was approved | 16.0.0 |
| 2020-03 | CT#87e | CP-200227 | 0008 | 2 | F | Reference Points | 16.1.0 |
| 2020-03 | CT#87e | CP-200019 | 0001 | 1 | F | HSS can be consumer of Nudr | 16.1.0 |
| 2020-03 | CT#87e | CP-200019 | 0002 | 1 | F | UserState retrieval | 16.1.0 |
| 2020-03 | CT#87e | CP-200019 | 0004 | 2 | B | PGW-C+SMF Information Notification Procedure | 16.1.0 |
| 2020-03 | CT#87e | CP-200019 | 0011 | - | B | IMEI Retrieval | 16.1.0 |
| 2020-03 | CT#87e | CP-200022 | 0006 | 4 | B | Introduce 5G SRVCC support | 16.1.0 |