3GPP TR 23.700-24 V0.7.0 (2020-08)

Technical Report

3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on support of the 5GMSG Service; (Release 17)

| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |
| --- | --- |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

Introduction	7

1	Scope	8

2	References	8

3	Definitions, symbols and abbreviations	9

3.1	Definitions	9

3.2	Symbols	9

3.3	Abbreviations	9

4	Scenarios	9

4.1	General	9

4.2	Scenario 1: Point-to-point message	11

4.2.1	Scenario 1.1: Message between 5GMSGS UEs	11

4.2.2	Scenario 1.2: Message between non-3GPP Controller UE and 5GMSGS Controlled UE	12

4.2.3	Scenario 1.3: Message between 5GMSGS Controller UE and non-3GPP Controlled UE	12

4.3	Scenario 2: Application-to-point message	13

4.3.1	Application-to-point scenarios overview	13

4.3.2	Scenario 2.1: Message from controller UE to Application Server	13

4.3.3	Scenario 2.2: Message from Application Server to Controlled UE	14

4.4	Scenario 3: Group message	15

4.4.1	Scenario 3.1: Group message between 5GMSGS UEs	15

4.4.2	Scenario 3.2: Group message between non-3GPP UEs and 5GMSGS UEs	16

4.5	Scenario 4: Broadcast message	17

4.5.1	Scenario 4.1: Message Broadcast from MSGin5G Server	17

4.5.2	Scenario 4.2: Message Broadcast initiated from 5GMSGS UE	17

4.5.3	Scenario 4.3: Message broadcast from non-3GPP Message Service to 5GMSGS UEs	18

5	Key Issues	18

5.1	Key Issue 1: support acknowledgement of delivery status	18

5.1.1	Description	18

5.1.2	Identified Gaps	19

5.2	Key Issue 2: Cell Broadcast Service in MSGin5GS	19

5.2.1	Description	19

5.2.1.1	Issue statement	19

5.2.1.2	Introduction to the Cell Broadcast Service	19

5.2.1.3	Analyses of the requirements	19

5.2.2	Identified Gaps	20

5.3	Key Issue 3: identifier(s) of the UE	21

5.3.1	Description	21

5.3.2	Identified Gaps	21

5.4	Key Issue 4: network triggering the UE	21

5.4.1	Description	21

5.4.2	Identified Gaps	21

5.5	Key Issue 5: Message transfer from 5GMSGS UE and Non-3GPP UE to MSGin5G Server	22

5.5.1	Description	22

5.5.2	Identified gaps	22

5.6	Key Issue 6: Message delivery from MSGin5G Server to Non-3GPP UE	22

5.6.1	Description	22

5.6.2	Identified gaps	23

5.7	Key Issue 7: group message	23

5.7.1	Description	23

5.7.2	Identified Gaps	24

5.8	Key Issue 8: group management	24

5.8.1	Description	24

5.8.2	Identified Gaps	24

5.9	Key Issue 9: Application-to-point message exchange	24

5.9.1	Description	24

5.9.2	Identified Gaps	25

5.10	Key Issue 10: broadcast area configuration	25

5.10.1	Description	25

5.10.2	Identified Gaps	26

5.11	Key Issue 11: mobility management of MSGin5G related information	26

5.11.1	Description	26

5.11.2	Identified Gaps	26

5.12	Key Issue 12: End-to-end latency	26

5.12.1	Description	26

5.12.2	Identified Gaps	27

5.13	Key Issue 13: Message store and forward	27

5.13.1	Description	27

5.13.2	Identified Gaps	28

5.14	Key Issue 14: Message Segmentation and Reassembly	28

5.14.1	Description	28

5.14.2	Identified Gaps	28

5.15	Key Issue 15: Message delivery from MSGin5G Server	29

5.15.1	Description	29

5.15.2	Identified Gaps	29

5.16	Key Issue 16: Charging	29

5.16.1	Description	29

5.16.2	Identified Gaps	29

5.17	Key Issue 17: Interconnection	30

5.17.1	Description	30

5.17.2	Identified Gaps	30

5.18	Key Issue 18: Roaming	30

5.18.1	Description	30

5.18.2	Identified Gaps	31

5.19	Key Issue 19: Efficient resource usage of the control plane and user plane	31

5.19.1	Description	31

5.19.2	Identified Gaps	32

6	Solutions	32

6.1	Solution 1: Payload size of 2048 octets in Broadcast	32

6.1.1	Description	32

6.1.2	Impacts on existing nodes and functionality	32

6.1.3	Solution evaluation	32

6.2	Solution 2: Registering 5GMSGS Client to MSGin5G Server	32

6.2.1	Description	32

6.2.1.1	General	32

6.2.1.2	5GMSGS Client Registration	33

6.2.1.3	5GMSGS Client De-Registration	34

6.2.2	Impacts on existing nodes and functionality	35

6.2.3	Solution evaluation	35

6.3	Solution 3: MSGin5G Triggering	35

6.3.1	Description	35

6.3.1.1	General	35

6.3.1.2	MSGin5G Triggering	35

6.3.2	Impacts on existing nodes and functionality	37

6.3.3	Solution evaluation	37

6.4	Solution 4: Group management using SEAL	38

6.4.1	General	38

6.4.2	Solution description for establishing a group	38

6.4.3	Solution description for adding UEs to the group or removing UEs from the group	38

6.4.4	Solution description for configuration of a maximum number of members in a group	38

6.4.5	Solution evaluation	38

6.5	Solution 5: 5GMSGS Group messaging	38

6.5.1	Description	38

6.5.1.1	General	38

6.5.1.2	5GMSGS Group messaging	38

6.5.1.3	5GMSGS client sends message to group	39

6.5.1.4	5GMSGS client receives message from group	41

6.5.1.5	Legacy 3GPP message client receives message from group	42

6.5.1.6	Non-3GPP message client receives message from group	42

6.5.2	Impacts on existing nodes and functionality	43

6.5.3	Solution evaluation	43

6.6	Solution 6: Charging	43

6.6.1	Description	43

6.7	Solution 7: MSGin5G Server message broadcast	44

6.7.1	Description	44

6.7.1.1	General	44

6.7.1.2	MSGin5G message broadcast	44

6.7.2	Impacts on existing nodes and functionality	46

6.7.3	Solution evaluation	46

7	Architectural requirements	46

7.1	Capability requirements	46

7.1.1	General	46

7.1.2	Requirements	46

8	Application architecture of the MSGin5G Service	47

8.1	Application architecture principles	47

8.2	Application architecture	47

8.3	Functional elements	48

8.3.1	General	48

8.3.2	MSGin5G Server	48

8.3.3	5GMSGS Client	49

8.3.4	MSGin5G Gateway 	49

8.3.5	Application Client	49

8.3.6	Application Server	49

8.3.7	3GPP Message Client	49

8.3.8	Non-3GPP Message Client	49

8.3.9	Non-3GPP Message Server	49

8.3.10	SEAL client	50

8.3.11	SEAL server	50

8.4	Reference Points	50

8.4.1	General	50

8.4.2	MSGin5G-1	50

8.4.3	MSGin5G-2	50

8.4.4	MSGin5G-3	50

8.4.5	MSGin5G-4	50

8.4.6	MSGin5G-5	51

8.4.7	MSGin5G-X	51

8.4.8	MSGin5G-Y	51

8.4.9	SEAL-C	51

8.4.10	SEAL-S	51

8.4.11	SEAL-UU	51

9	Overall Evaluation	51

10	Conclusions	52

Annex A: 5G Architecture considerations	53

Annex B: Change history	55

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

# 1	Scope

The present document studies solutions to satisfy the requirements for MSGin5G Service (message service for MIoT over 5G System). This TR will study the application architecture needed to support MSGin5G  service over 5G system, based on the stage 1 requirements specified in 3GPP TS 22.262 [2].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.262: "Message Service within the 5G System".

[3]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[4]	GSMA PRD RCC.07: "RCC.07 Rich Communication Suite 9.0 Advanced Communications Services and Client Specification v10.0".

[5]	OMA OMA-ERELD-LightweightM2M-V1_1-20180612-C: "Enabler Release Definition for LightweightM2M".

[6]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[7]	3GPP TS 23.038: "Alphabets and language-specific information".

[8]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); protocol specification".

[9]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals".

[10]	3GPP TS 23.502: "Procedures for the 5G System".

[11]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[12]	ATIS-0700008.v002: "Cell Broadcast Entity (CBE)-to-Cell Broadcast Center (CBC) Interface Specification".

[13]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[14]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Controller UE: usually a smart device or an application in the terminal that can send control message instructions to machine equipment. A controller terminal can control multiple controlled terminals.

Controlled UE: an IoT equipment which can access the network through a variety of technologies and network slicing. A controlled terminal can be controlled by a number of controller terminals.

Non-3GPP Message: the message supported in a message service which is not defined by 3GPP (e.g. RCS message as specified in GSMA PRD RCC.07 [4], OMA LWM2M message as specified in OMA OMA-ERELD-LightweightM2M [5]).

MSGin5G service capabilities: the subset of MSGin5G service available to access and/or communicate with other UEs or application servers. The whole set of MSGin5G service includes point-to-point messaging, application-to-point messaging, group messaging and broadcast messaging.

For the purpose of the present document, the following terms given in 3GPP TS 22.262 [2] apply:

MSGin5G Server

MSGin5G Gateway

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

# 4	Scenarios

[Editor's Note: scenarios and service level requirements from Stage 1]

## 4.1	General

Four message communication models are specified in 3GPP TS 22.262 [2], including:

a) point-to-point message

b) application-to-point message

c) group message

d) broadcast message

In this document, three logical UE types will be used to describe the scenarios of each message communication model from technical point of view, The UE types are classified depending on the type of underlying transport services supported, as follows:

5GMSGS UE type - originating/terminating only 5GMSGS transport.

Legacy 3GPP UE type - originating/terminating only legacy 3GPP transports, e.g. SMS, NIDD, broadcast, etc.

Non-3GPP UE type - originating/terminating only non-3GPP transports.

NOTE:	Support for the 5GMSGS transport protocol is not equivalent with support for MSGin5G Application Services. All UE types described in this clause support MSGin5G Application Services

For each of these types, support of the corresponding messaging transports is exclusive, i.e. 5GMSGS UE type supports only 5GMSGS transport and does not support legacy 3GPP transports or non-3GPP transports. The correspondence between UE types A/B/C/D described in 3GPP TS 22.262 [2] and the UE types described in this Technical Report:

-	The 5GMSGS type corresponds to both types A and B.

-	The Legacy 3GPP type is derived from type C and supports NIDD, broadcast, etc. in addition to SMS. Legacy 3GPP UE type may also originate MSGin5G Application Service messaging.

-	The Non-3GPP type is derived from type D.

This Technical Report will focus on the common key issues of MSGin5G Service. The requirements specified in 3GPP TS 22.262 [2] and assumptions based on 3GPP TS 22.262 [2] apply to the scenarios。

1)	The following applies to the point-to-point message scenario:

-	Typical use case of point-to-point message is remote control of intelligent terminals (see 3GPP TS 22.262 [2])

-	[R-5.2.2-001] The MSGin5G Service shall support Mobile Originated Mobile Terminated (MOMT) messaging, i.e. messages are originated and terminated at UEs.

-	[R-5.2.2-002] The MSGin5G Service shall support addressing the UE by IMSI/MSISDN or IMEI.

The following assumptions can be made based on 3GPP TS 22.262 [2]:

-	Most UEs in this scenario do not have the limitation of power, and are online most time.

2) The following applies to the application-to-point message scenario:

-	Typical use cases of application-to-point message are monitoring data reporting and remote control.  (see 3GPP TS 22.262 [2])

-	[R-5.3.2-001] The MSGin5G Service shall support Mobile Originated Application Terminated (MOAT) messaging, i.e. messages are originated at a UE and terminated at an application server in the network.

-	[R-5.3.2-002] The MSGin5G Service shall support Application Originated Mobile Terminated (AOMT) messaging, i.e. messages are originated at an application server in the network and terminated at a UE.

-	[R-5.3.2-003] The MSGin5G Service shall support Application Originated Mobile Terminated messaging service with max latency of 10 seconds while maintaining battery life of at least 3 months for small data traffic once every hour and typical sized IOT battery [200-500mAh].

-	[R-5.1.2-006]	The MSGin5G Service shall support a server in the network triggering the UE to perform an action (e.g. wake up and establish a PDN connection).

Based on requirements [R-5.3.2-003] and [R-5.1.2-006] listed above, the following assumptions can be made:

-	UE Power Saving Mode specified in 3GPP TS 23.682 [3] is needed to be considered.

-	Requirement [R-5.2.2-002] listed above will also apply.

3) The following applies to the group message scenario:

-	Typical use case of group message is smart home.  (see 3GPP TS 22.262 [2]).

-	[R-5.4.2-001] The MSGin5G Service shall support group message communication, i.e. a UE sends a message to a group of UEs. All the members in a group can send messages. The UEs in a group can be located in different geographical areas.

The following assumptions can be made based on 3GPP TS 22.262 [2]:

-	All kinds of UE (e.g. application server, devices with or without power and computation limitation, devices always online or usually without IP connection) may use group message. Only a limited number of UEs in the group have the ability of group management.

-	Requirement [R-5.2.2-002] listed above will also apply to both originating UE and terminating UE.

4) The following applies to the broadcast message scenario:

-	[R-5.5.2-002] The MSGin5G Service shall support an authorized application server or UE to send a broadcast message to all the UEs within a specific area which is configured according to application policy.

-	The assumptions of UE type in the group message scenario will also apply.

-	Requirement [R-5.2.2-002] listed above applies to the originating UE.

## 4.2	Scenario 1: Point-to-point message

### 4.2.1	Scenario 1.1: Message between 5GMSGS UEs

This scenario describes the case where Controller UE1 (e.g. a mobile handset terminal 1 or an application client APP 1 in a mobile handset Terminal 1) communicates with Controlled UE2 (e.g. wearable devices Terminal 2 like smart watch) or Controlled UE C. Both UE1 and UE2 are 5GMSGS type and connected to a same 5G system, use MSGin5G service and served by same MSGin5G application layer. UE C is connected to the 5G system that supports SMS but does not support the MSGin5G Service. APP2 in the terminal 1 may also MSGin5G capable, but not used in this communication.

NOTE 1:	MSGin5G application layer may consist of numerous functions, e.g. MSGin5G Server. The application architecture to support the scenarios will be studied.

NOTE 2: The UE1, UE2, etc in this Clause mean the party 1 and party 2 in a messaging procedure. It is not related t to a specific UE type.

The message will be routed via the 5G system from UE1 to UE2 or UE C, or vice versa. The message may also be first routed to the MSGin5G application layer and then forwarded to the other party. The scenario is illustrated in figure 4.2.1-1.

Figure 4.2.1-1: Message between 5GMSGS clients

Editor's Note: The Figure 4.2.1-1 merges the changes from S6-200614 and S6-200585. The UE C needs to be updated according to S6-200614.

The scenario consists of the following aspects:

-	If message is originated by authorized UE1, the MSGin5G server identifies the identity of UE2 or UE C and forwards the MSGin5G message to UE2 or UE C. UE2 or MSGin5G server may send the delivery status to UE1.

-	If message is originated by authorized UE2, the MSGin5G service identifies the identity of UE1 (e.g. a specific APP1in terminal 1) and forwards the MSGin5G message to UE1. UE1 or MSGin5G server may send the delivery status to UE2 or UE C.

### 4.2.2	Scenario 1.2: Message between non-3GPP Controller UE and 5GMSGS Controlled UE

This scenario describes the case where Controller UE1 (e.g. a mobile handset Terminal 1, or an application client APP 1 in a mobile handset Terminal 1) communicates with Controlled UE2 (e.g. wearable device Terminal 2 like a smart watch) by using non-3GPP message service (e.g. RCS message service specified in GSMA PRD RCC.07 [4]), and UE2 uses MSGin5G service and is 5GMSGS transport capable..

UE1 is connected to a non-3GPP message server and then connected to MSGin5G application layer via MSGin5G gateway. UE2 connected the 5G system directly and served by MSGin5G application layer. The scenario is illustrated in figure 4.2.2-1.

NOTE:	The MSGin5G gateway facilitates the translation between the MSGin5G Service and non-3GPP message service. The application functional aspect and procedure in MSGin5G gateway will be studied.

Figure 4.2.2-1: Message between non-3GPP Controller UE and 5GMSGS Controlled UE

The scenario consists of the following aspects:

-	UE 1 is authorized by the MSGin5G service via MSGin5G Gateway.

-	The MSGin5G gateway records the mapping between the identity in MSGin5G service and the identity in non-3GPP message service for both UE1 and UE2.

-	If message is originated by authorized UE1, non-3GPP message server forwards the message to MSGin5G gateway. MSGin5G server identifies the UE2 and forward the MSGin5G message to UE2. UE2 or MSGin5G server may send the delivery status to UE 1.

For the case where Controlled UE2 (e.g. wearable device Terminal 2 like smart watch) communicates with Controller UE1 (e.g. a mobile handset Terminal 1, or an application client APP 1 in a mobile handset Terminal 1) that uses non-3GPP message (e.g. RCS message service specified in GSMA PRD RCC.07 [4]) .

-	If message is originated by authorized UE2, the MSGin5G service forwards the message to UE1 (mobile handset terminal 1 or a specific application in terminal 1) in the MSGin5G service, and the MSGin5G gateway forward the message to UE1. UE1 or MSGin5G server may send the delivery status to UE 2.

### 4.2.3	Scenario 1.3: Message between 5GMSGS Controller UE and non-3GPP Controlled UE

This scenario describes the case where Controller UE1 (e.g. a mobile handset Terminal 1, or an application client APP 1 in a mobile handset Terminal 1) communicates with Controlled UE B  (e.g. wearable devices Terminal 2 like smart watch) by using MSGin5G service, and UE2 uses non-3GPP message service (e.g. OMA LWM2M specified in OMA OMA-ERELD-LightweightM2M [5]).

UE1 connected to 5G system and served by MSGin5G application layer. UE2 served by non-3GPP message server. The MSGin5G message and non-3GPP message will be transferred to each other by the MSGin5G gateway. The scenario is illustrated in figure 4.2.3-1.

Figure 4.2.3-1: Message between 5GMSGS Controller UE and non-3GPP Controlled UE

The scenario consists of the following aspects:

-	UE2 is authorized by the MSGin5G service via MSGin5G Gateway.

-	The MSGin5G gateway records the mapping between the identity in MSGin5G service and the identity in non-3GPP message service for both UE1 and UE2.

-	If message is originated by authorized UE1, the MSGin5G server forwards the message to UE2 in the MSGin5G service, and the MSGin5G gateway forwards the message to UE2. UE2 or MSGin5G server may send the delivery status to UE1.

For the case where Controlled UE2 (e.g. wearable device Terminal 2 like smart watch) that uses non-3GPP message communicates with Controller UE1 (e.g. a mobile handset Terminal 1, or an application client APP 1 in a mobile handset Terminal 1).

-	If message is originated by authorized UE2, the MSGin5G server forwards the MSGin5G message to the MSGin5G gateway. MSGin5G server identifies the identity of UE1and forward the non-3GPP message to UE1. UE1 or MSGin5G server may send the delivery status to UE2.

NOTE:	The scenario is out of scope if both UE1 and UE2 use non-3GPP message.

## 4.3	Scenario 2: Application-to-point message

### 4.3.1	Application-to-point scenarios overview

In the application-to-point scenarios MSGin5G messages originate from an application, which may reside on Controller UEs. Controller UEs can be 5GMSGS UE type, Non-3GPP UE type or a combination of them. In all cases the message is sent to the MSGin5G application layer and terminates at the Application Server. All UEs send messages to the application layer via the 5G access network and the Application Server is connected to the MSGin5G application layer directly.

The Application Server processes the received messages and this could result, depending on the service requirements, in a message that is sent by the Application Server and which terminates at a Controlled UE. Controlled UEs can be 5GMSGS UE type, Legacy 3GPP UE type, Non-3GPP UE type or a combination of them. .

An example of such a service is where electric scooters send their status (e.g. location and battery level) at a regular interval to the server (APP1) and when such a scooter is rented by an individual (APP2) the server sends a message to that scooter to unlock it.

### 4.3.2	Scenario 2.1: Message from controller UE to Application Server

This scenario describes the case where a Controller UE (e.g. a mobile handset terminal or an application client APP 1 in a mobile handset terminal) communicates with the Application Server. 5GMSGS UE,  or Non-3GPP UE is connected to the 5G system, UE A uses the MSGin5G service and is served by the MSGin5G application layer. UE D uses  non-3GPP message service and is connected to MSGin5G application layer via MSGin5G gateway.

5GMSGS UE is connected to the 5G access network and sends messages via the 5GMSGStransport to the MSGin5G application layer.

Legacy 3GPP UE is connected to the 5G access network and sends messages via legacy 3GPP transports, e.g. SMS, NIDD, to the MSGin5G application layer.

Non-3GPP UE sends its non-3GPP messages via the MSGin5G Gateway to the MSGin5G application layer.

NOTE:	MSGin5G application layer may consist of numerous functions, e.g. MSGin5G Server. The application architecture to support the scenarios will be studied.

The message will be routed via the 5G system from the UE to the Application Server. The scenario is illustrated in figure 4.3.2-1.

Figure 4.3.2-1: Message from controller UE to Application Server

The scenario consists of the following aspects:

-	If a message is originated by authorized 5GMSGS UE or Non-3GPP UE, the MSGin5G service identifies the identity of the Application Server and forwards the MSGin5G message to this Application Server. The Application Server may send the delivery status to the originating UE via the MSGin5G service.

### 4.3.3	Scenario 2.2: Message from Application Server to Controlled UE

This scenario describes the case where Application Server communicates with Controlled UEs using the MSGin5G service. The controlled UEs can be 5GMSGS UE type, Legacy 3GPP UE type, Non-3GPP UE type or a combination of them.

The Application Server is connected directly to the MSGin5G application layer.

5GMSGS UE is connected to the 5G access network and receives messages via the MSGin5G service.

Legacy 3GPP UE is connected to the 5G access network and receives messages via legacy 3GPP transports, e.g. SMS, NIDD, broadcast.

Non-3GPP UE receives its non-3GPP messages via the MSGin5G Gateway.

The scenario is illustrated in figure 4.3.3-1.

NOTE:	The MSGin5G gateway facilitates the translation between the MSGin5G Service and non-3GPP message service. The application functional aspect and procedure in MSGin5G gateway will be studied.

Figure 4.3.3-1: Message from Application Server to MSGin5G Controlled UEs

The scenario consists of the following aspects:

-	The Application Server is authorized to use the MSGin5G application layer.

-	If a message is originated by an authorized Application Server and should terminate at 5GMSGS UE or Legacy 3GPP UE, then MSGin5G application layer delivers the message to the UE. The UE may send the delivery status to the Application Server.

-	The MSGin5G Gateway records the mapping between the identity in MSGin5G service and the identity in non-3GPP message service for Non-3GPP UE.

-	If a message is originated by an authorized Application Server and should terminate at Non-3GPP UE then the MSGin5G service identifies the Non-3GPP UE and forwards the MSGin5G message to MSGin5G Gateway. The MSGin5G Gateway converts the message format and delivers the message to Non-3GPP UE. Non-3GPP UE or the MSGin5G Gateway may send the delivery status to the Application Server.

## 4.4	Scenario 3: Group message

### 4.4.1	Scenario 3.1: Group message between 5GMSGS UEs

This scenario describes the case where 5GMSGS UE1 (e.g. a mobile handset terminal 1 or an application client APP 1 in a mobile handset Terminal 1) communicates with a group of UEs (5GMSGS UEs or legacy 3GPP UEs or non-3GPP UE). All UEs may be in different geographical area.

NOTE:	MSGin5G server may consist of numerous functions, e.g. MSGin5G Server group function. The application architecture to support the scenarios will be studied.

The message will be first routed to the MSGin5G server and then forwarded to all other parties. The scenario is illustrated in figure 4.4.1-1.

Figure 4.4.1-1: Group message between 5GMSGS UEs

Editor's Note: The Figure 4.4.1-1 merges the changes from S6-200614 and S6-200521. The UEs name need to be updated according to S6-200614.

The scenario consists of the following aspects:

-	all  in group can communicate with each other, which means the message sent by a 5GMSGS UE will be received by all the other UEs in the group, either directly from MSGin5G server (if 5GMSGS UEs) or via MSGin5G gateway (if non-3GPP UEs).

NOTE:	Group message between UE C and UE D is also supported when the group is hosted on 5G MSG Server.

### 4.4.2	Scenario 3.2: Group message between non-3GPP UEs and 5GMSGS UEs

This scenario describes the case where non-3GPP UE1 (e.g. a mobile handset Terminal, or an application client APP 1 in a mobile handset Terminal 1) communicates with a group of UEs by using non-3GPP message (e.g. RCS message service specified in GSMA PRD RCC.07 [4]), and some of UEs in this group use MSGin5G service. All UEs in the group may be in different geographical area.

UE1 connect to a non-3GPP message server and then connected to MSGin5G server via MSGin5G gateway. All 5GMSGS UEs connected the 5G system directly and served by MSGin5G server. The scenario is illustrated in figure 4.4.2-1.

NOTE:	The MSGin5G gateway facilitates the translation between the MSGin5G Service and non-3GPP message service. The application functional aspect and procedure in MSGin5G gateway will be studied.

Figure 4.4.2-1: Message from non-3GPP Message Service to MSGin5G group

The scenario consists of the following aspects:

-	all UEs in group can communicate with each other, which means the message sent by a non-3GPP UE will be received by all the other 5GMSGS UEs in the group, and all the other 5GMSGS UEs can send the message to a non-3GPP UE.

## 4.5	Scenario 4: Broadcast message

### 4.5.1	Scenario 4.1: Message Broadcast from MSGin5G Server

This scenario describes the case where an APP Server in the network communicates with Controlled UEs by using MSGin5G service via broadcast.

The message will be routed via the 5G system from the APP Server to UEs. The scenario is illustrated in figure 4.5.1-1.

The recipient UE is connected to the same 5G system and support broadcast.

Figure 4.5.1-1: Message Broadcast from APP Server to UEs

Editor's Note: The Figure 4.4.1-1 merges the changes from S6-200612 and S6-200614. The UEs name need to be updated according to S6-200614.

The scenario consists of the following aspects:

-	The message is originated by authorized APP Server and the APP Server also provides the service area information.

-	The MSGin5G Server forwards the MSGin5G message to the 5G System, which takes care of broadcasting the message in the designated service area.

-	UEs that support the MSGin5G Service and are located in the service area and configured to receive the message will receive it via broadcast.

### 4.5.2	Scenario 4.2: Message Broadcast initiated from 5GMSGS UE

This scenario describes the case where Controller UE1 (e.g. a mobile handset Terminal 1, or an application client APP 1 in a mobile handset Terminal 1) communicates with Controlled UEs 2 via broadcast.

The message will be routed via the 5G system from the APP in UE1 to UEs 2. The scenario is illustrated in figure 4.5.2-1.

Figure 4.5.2-1: Message Broadcast from APP in UE1 to UEs2

Editor's Note: The Figure 4.4.1-1 merges the changes from S6-200612 and S6-200614. The UEs name need to be updated according to S6-200614.

The scenario consists of the following aspects:

-	The message is originated by authorized APP in UE1 and the APP also provides the service area information.

-	The MSGin5G Server forwards the MSGin5G message to the 5G System, which takes care of broadcasting the message in the designated service area.

-	UEs that support the MSGin5G Service and are located in the service area and configured to receive the message will receive it via broadcast.

### 4.5.3	Scenario 4.3: Message broadcast from non-3GPP Message Service to 5GMSGS UEs

This scenario describes the scenario where Non-3GPP UE1  (e.g. OMA LWM2M specified in OMA OMA-ERELD-LightweightM2M [5]) delivers a broadcast message by using non-3GPP Message service to 5GMSGS UEs2 (e.g. a mobile handset terminal 1 or an application client APP 1 in a mobile handset Terminal 1)  to 5GMSGS UEs2which is configured according to application policy.

5GMSGS UEs2 connected to 5G system and served by MSGin5G server. The MSGin5G message and non-3GPP message will be translated by the MSGin5G gateway. The scenario is illustrated in figure 4.5.3-1.

Figure 4.5.3-1: Message broadcast from non-3GPP message service to 5GMSGS UEs 2

The scenario consists of the following aspects:

-	If broadcast message is originated by authorized Non-3GPP UE1, the MSGin5G Server broadcasts the message to all the UEs. 5GMSGS UEs may send the delivery status back to Non-3GPP UE1 or APP Server.

# 5	Key Issues

## 5.1	Key Issue 1: support acknowledgement of delivery status

### 5.1.1	Description

The requirement of supporting delivery status is specified in requirement [R-5.1.2-004] of 3GPP TS 22.262 [2].

In many use cases of MSGin5G service, the message originator cares about delivery status of some specific messages, or makes further decision based on the delivery status of previous messages. In some other use cases, the message originator may not care about delivery messages, e.g. a smart device in a group may not care whether a message it sent to the group is received by some other smart devices in the group, or in case of broadcast to a large number of devices an acknowledgement from receiving devices may not be useful.

Furthermore, if non-3GPP message is used by some UEs, delivery status of MSGin5G service is needed to be mapped to delivery status of non-3GPP message.

Issues include:

-	How to identify if delivery status is required for some specific message.

-	How to require the delivery status of some specific message.

-	How the delivery status of MSGin5G service interwork with non-3GPP message services.

### 5.1.2	Identified Gaps

The following gaps have been identified to fulfill the delivery status acknowledgement:

-	The protocol requirements for delivery status acknowledgement and sending a delivery status acknowledgement shall be studied.

-	If delivery status acknowledgement is supported by the non-3GPP message service, the procedure of delivery status acknowledgement interworking in the MSGin5G gateway will be defined.

-	If delivery status acknowledgement is not supported by the non-3GPP message service, then if the MSGin5G gateway or MSGin5G server will send the delivery status acknowledgement on behalf of the non-3GPP UE needs to be defined.

## 5.2	Key Issue 2: Cell Broadcast Service in MSGin5GS

### 5.2.1	Description

#### 5.2.1.1	Issue statement

This key issue analyses if the Cell Broadcast Service as specified in 3GPP TS 23.041 [6] fulfills the requirements of the broadcast component of MSGin5GS as specified in 3GPP TS 22.262 [2].

#### 5.2.1.2	Introduction to the Cell Broadcast Service

The Cell Broadcast Service supports unacknowledged broadcasting of a text message in one or more cells. The message consists of Warning Message Content, Message Identifier, Serial Number and Data Coding Scheme parameters.

The Message Identifier indicates the source or type of content and together with the Serial Number identifies the message uniquely. The Data Coding Scheme indicates how the Warning Message Content is encoded.

The Warning Message Content NG-RAN IE contains the user information and is a string of maximum 9600 octets. This IE contains a maximum of 15 CBS-Message-Information-Pages of 82 octets each which results in 1230 octets, and not in 9600 octets. It is not specified how 9600 octets can be encoded.

The message is broadcast over the radio interface in SIB8; no data is sent over the User Plane (see 3GPP TS 38.331 [8]). The cmas-indication in the paging message is used to notify the UEs that SIB1 contains the scheduling information for receiving SIB8. The paging message is broadcast with a periodicity of 40 ms and SIB1 has a periodicity of 80 ms. SIB8 is broadcast with a configurable periodicity and 80 ms, 160 ms, 320 ms, and 640 ms are possible values. SIB8 may be segmented when it is broadcast over the air interface, when the maximum size of SIB8 is exceeded. This means that 200 ms is the minimum latency until the UE can start receiving the SIB8. The duration for receiving SIB8 depends on the total size of SIB8.

#### 5.2.1.3	Analyses of the requirements

3GPP TS 22.262 [2] contains general requirements in subclause 5.1.2 and broadcast message specific requirements in subclause 5.5.2. Other requirements are not related to message broadcast. These requirements are copied here (in italics) and analysed individually.

[R-5.1.2-001] The MSGin5G Service shall support UE sending and receiving a text or data message with end-to-end latency of less than 500 ms.

The latency for broadcasting SIB8 is at least 200 ms.

[R-5.1.2-002] The MSGin5G Service shall support variable size of payload of text and data with a maximum of 2048 bytes, and support segmented transmission if the content is larger than the maximum payload length of a message.

Cell Broadcast allows encoding of a payload of up to 1230 octets, but it is not specified how 2048 octets can be encoded, even though the maximum size of the payload string is specified to be 9600 octets.

[R-5.1.2-003] The MSGin5G Service shall support delivery of a message to a specific application in the terminated UE. This message contains the contents that can be handled by the specific application.

The Data Coding Scheme IE can be used to indicate that the payload is (U)SIM specific and shall be processed by an application instead of being displayed immediately (see 3GPP TS 23.038 [7]).

[R-5.1.2-004] The MSGin5G Service shall support acknowledgement of delivery status (success, failure) of a message and indication of reason if the delivery is failed.

The NG-RAN nodes report successful scheduling of broadcast to the CBC (not of failed scheduling).

[R-5.1.2-005] The MSGin5G Service shall support storage of a message if a UE is unavailable (disconnected or power off) for future delivery once the UE becomes available.

Cell Broadcast messages are repeatedly broadcast for as long as the message is valid.

[R-5.1.2-006] The MSGin5G Service shall support a server in the network triggering the UE to perform an action (e.g. wake up and establish a PDN connection).

Cell Broadcast uses the cmas-indication in the paging message to notify the UE to obtain SIB1 which contains the scheduling information to obtain SIB8 and the Warning Message Content NG-RAN IE is used by the application to perform an action.

[R-5.1.2-007] The MSGin5G Service shall support a UE sending and receiving messages via a MSGin5G Gateway.

The Cell Broadcast Centre (see 3GPP TS 23.041 [6]) could be an instantiation of a MSGin5G Gateway for broadcasting messages. Cell Broadcast does not support non-3GPP access.

[R-5.1.2-008] The MSGin5G Service shall support the mobility of a UE (i.e. the UE can still send/receive messages when it changes the location of network access).

Cell Broadcast broadcasts SIB8 repeatedly in one or more cells and UEs will receive that as long as they remain in coverage of those cells.

[R-5.5.2-001] The MSGin5G Service shall support broadcasting a text or data message with end-to-end latency less than [500] ms.

This requirement is identical to requirement R-5.1.2-001 above from a broadcast perspective.

[R-5.5.2-002] The MSGin5G Service shall support an authorized application server or UE to send a broadcast message to all the UEs within a specific area which is configured according to application policy.

3GPP TS 23.041 [6] specifies a Cell Broadcast Entity which serves as an entity where the broadcast message is generated. CBE functionality and an interface towards the Cell Broadcast Centre are not specified.

### 5.2.2	Identified Gaps

If Cell Broadcast technology is used to support the broadcast message in the MSGin5G Service then the following requirements apply:

-	It needs to be specified how a maximum of 2048 octets can be encoded in the Warning Message Content NG-RAN IE.

-	The architecture for the MSGin5G architecture contains a Cell Broadcast Centre and the interface between that Cell Broadcast Centre and the MSGin5G Server may need to be specified.

## 5.3	Key Issue 3: identifier(s) of the UE

### 5.3.1	Description

The requirement of identifying the UE is specified in requirement [R-5.2.2-002] of 3GPP TS 22.262 [2]. It also applies to other three message scenarios according to the assumptions in clause 4.1.

Different MSGin5G enabled applications in a mobile handset Terminal shall be identified by MSGin5G services and the requirement is specified in requirement [R-5.2.2-003] of 3GPP TS 22.262 [2]. It also applies to other three message scenarios according to the assumptions in clause 4.1.

Furthermore, if 5GMSG UE communicates with non-3GPP message UE, the identifiers of non-3GPP UE shall be mapped with identifier of 5GMSG UE.

Issues include:

-	How to uniquely identify a UE in MSGin5G service, the UE may include MSGin5G enabled devices, MSGin5G enabled application in a device and non-3GPP message device.

-	There may be many kinds of non-3GPP message services, whether it is necessary to establish a unique mechanism to map the non-3GPP identifier to 5GMSG identifier, and how to establish the mechanism.

### 5.3.2	Identified Gaps

The following gaps have been identified to fulfil the identifier requirements:

-	The IMSI/MSISDN or IMEI may not suitable to identify all kind of UEs, e.g. the applications in a mobile handset. The following studies are needed:

-	Whether the existing identifier(s) can fulfil the requirements.

-	Whether new identifier(s) is needed to be specified.

-	If the identifier(s) of UE is assigned by MSGin5G services, the procedure of generating, assigning and managing the identifier(s) are needed to be defined.

-	The procedure of managing both the MSGin5G identifiers(s) and identifier(s) of non-3GPP message services in MSGin5G Gateway is needed to be defined.

## 5.4	Key Issue 4: network triggering the UE

### 5.4.1	Description

The requirement of network triggering the UE is specified in requirement [R-5.1.2-006] of 3GPP TS 22.262 [2]. It applies to all message scenarios.

Some performance requirements also apply to the network triggering mechanism, i.e. the requirement [R-5.5.2-001] of 3GPP TS 22.262 [2].

Issues include:

-	Whether the existing network triggering mechanism, i.e. network triggered service request specified in clause 4.2.3.3 of 3GPP TS 23.502 [10], can fulfil the network triggering requirement specified in 3GPP TS 22.262 [2].

-	When UE uses non-3GPP message service, whether it is necessary for MSGin5G service to trigger the UE and how MSGin5G service triggers the UE.

### 5.4.2	Identified Gaps

The following gaps have been identified to fulfil the identifier requirements:

-	If network triggered service request specified in clause 4.2.3.3 of 3GPP TS 23.502 [10] is used to trigger the UE, potential impact to on 5G system is needed to be studied.

-	Specify an MSGin5G trigger procedure on top of 5GS if needed.

-	How to trigger non-3GPP message UE and the related procedure are needed to be studied.

## 5.5	Key Issue 5:	Message transfer from 5GMSGS UE and Non-3GPP UE to MSGin5G Server

### 5.5.1	Description

5GMSGS UE is a Controller UE that sends messages in MSGin5G message format to the MSGin5G Server.

Non-3GPP UE is a Controller UE that sends messages in a non-3GPP message format via the MSGin5G Gateway to the MSGin5G Server.

The 5G Network provides data delivery methods over the control plane (i.e. via N1) and over the user plane (i.e. via N3). Application Servers access control plane and user plane data delivery features via N6 or via exposed APIs (e.g. T8 and Nnef).

### 5.5.2	Identified gaps

-	Whether the MSGin5G Service uses existing control plane data delivery methods, existing user plane data delivery methods or both.

-	Whether the MSGin5G Service requires new or enhanced data delivery methods.

-	Whether the MSGin5G Service uses existing methods (e.g. N6, T8, and Nnef) in order to access user plane and/or control plane data delivery methods.

-	Whether the MSGin5G Service requires new or enhanced methods in order to access data delivery methods.

## 5.6	Key Issue 6:	Message delivery from MSGin5G Server to Non-3GPP UE

### 5.6.1	Description

The MSGin5G Server supports messages in the MSGin5G message format and needs to be able to deliver messages to Non-3GPP UE type devices that do not support the MSGin5G message format, but support a non-3GPP message format.

The MSGin5G Gateway acts as a gateway as shown in figure 5.6.1-1 and takes care of message delivery via the 5G System.

Figure 5.6.1-1 also shows message delivery (and reception) to Device E. Device E is a device that does not support the MSGin5G Service and is not connected to the 5G System but is connected to the MSGin5G Server, via the MSGin5G Inter-Working Function (IWF), but has otherwise similar capabilities and functionality as Non-3GPP UE has.

Figure 5.6.1-1: MSGin5G Gateway as gateway

The MSGin5G Gateway needs to support the capability of message delivery to Non-3GPP UE in point-to-point, point-to-group and broadcast scenarios. The MSGin5G Server supports this same capability for delivery of messages to 5GMSGS UE in the same scenarios.

### 5.6.2	Identified gaps

-	The interface between the MSGin5G Server and the MSGin5G Gateway needs to be specified.

-	The interface between the MSGin5G Server and the MSGin5G IWF needs to be specified and it needs to be studied if the interface towards the MSGin5G Gateway can be the same as the interface towards the MSGin5G IWF.

NOTE: Functionality of the MSGin5G Gateway and MSGin5G IWF beyond the interface to the MSGin5G Server are out of scope of the study.

## 5.7	Key Issue 7: group message

### 5.7.1	Description

The requirement of group message is specified in requirement [R-5.4.2-001] of 3GPP TS 22.262 [2]:

-	[R-5.4.2-001] The MSGin5G Service shall support group message communication, i.e. a UE sends a message to a group of UEs. All the members in a group can send messages. The UEs in a group can be located in different geographical areas.

The requirements of roaming and interconnection specified in 3GPP TS 22.262 [2] are also applied:

-	[R-8.2-001] The MSGin5G Service shall support a roaming UE sending and receiving messages.

-	[R-9.2-001] The MSGin5G Service shall support an UE within an operator sending a message to another UE within another operator.

The following conclusion can be made based on the requirements above:

-	The UEs in a group may locate in different PLMNs or roam to VPLMN.

According to requirement [R-5.1.2-007] of 3GPP TS 22.262 [2], non-3GPP message UE may also send and receive group message via MSGin5G gateway.

Issues include:

-	How to support different UEs sending and receiving group message in an MGSin5G group. The UE may include 5GMSGS UE located in HPLMN, 5GMSGS UE located in VPLMN, 5GMSGS UE interconnected from other PLMN and non-3GPP UE,

### 5.7.2	Identified Gaps

The following gaps have been identified to fulfil the identifier requirements:

-	The procedures of group message, including originating and terminating group message for different UEs, are needed to be specified.

-	The related functional requirements, e.g. identifier of group, authorization of UEs, etc, are needed to be specified.

## 5.8	Key Issue 8: group management

### 5.8.1	Description

The requirement of group management is specified in requirement [R-5.4.2-002] of 3GPP TS 22.262 [2]. However, it is not specified that who (e.g. all UEs or some specific UEs in the group, the MSGin5G server) has the capability of group management.

Issues include:

-	Analyze who can have the capability of group management from the technical perspective.

-	How to generate, manage and synchronize the group information on UE and MSGin5G server. The UE may include 5GMSGS UE located in HPLMN, 5GMSGS UE located in VPLMN, 5GMSGS UE interconnected from other PLMN and non-3GPP UE,

-	How to manage non-3GPP UE in a MSGin5G group message group.

### 5.8.2	Identified Gaps

The following gaps have been identified to fulfil the identifier requirements:

-	Whether the existing mechanism, e.g. SEAL specified in 3GPP TS 23.434 [9] can fulfil the group management requirements specified in 3GPP TS 22.262 [2]. If gaps are identified, the following studies are needed:

-	The group information used for MSGin5G group management.

-	The procedures of MSGin5G group management.

-	The group management interworking in the MSGin5G gateway are needed to be defined.

-	Technical requirements caused by the characters of UEs are needed to be fulfilled.

-	UE may have the limitation of resource, some specified mechanism of managing the group data in the UE are needed.

-	Long latency may exist between UE and MSGin5G server due to the UE may be offline or in Power Saving Mode specified in 3GPP TS 23.682 [3]. The group data in the UE and the MSGin5G server may not be synchronized on time, so some abnormal scenarios are needed to be specified.

## 5.9	Key Issue 9: Application-to-point message exchange

### 5.9.1	Description

The requirements of application-to-point message exchange are specified in requirement [R-5.3.2-001] and [R-5.3.2-002] of 3GPP TS 22.262 [2].

Various application servers may use the MSGin5G Services, and different types of UE, e.g. 5GMSGS UE, legacy 3GPP UE and non-3GPP UE, may exchange Application-to-point message with those application servers.

Issues include:

-	For the MOAT message,

-	How to uniquely identify a logical application server in MSGin5G service. The logical application servers may be connected to the MSGin5G application layer in the 5G core network directly or through the NEF.

NOTE:	The logical application server is used for a specific user service. There is only a single logical application server and many applications clients in UEs for a specific user service. Multiple logical application servers, each for a specific user service, may connect to a MSGin5G Server.

-	How to share the contact information of logical application server with different types of UE.

-	How to interact between MSGin5G server and different logical application servers in MOAT scenario.

-	For the AOMT message

-	How to share the contact information of different types of UE with different logical application servers. The UEs can be 5GMSGS UE, legacy 3GPP UE and non-3GPP UE. The logical application servers may be connected to the MSGin5G application layer in the 5G core network directly or through the NEF.

-	How to interact between MSGin5G server and different logical application servers in AOMT scenario.

### 5.9.2	Identified Gaps

The following gaps have been identified to fulfil the identifier requirements:

-	Whether the identifier(s) of UE also suitable for identifying the different kinds of logical application servers.

-	The procedure of sharing contact information between different kinds of logical application servers and UEs is needed to be defined.

-	The interaction procedure between different kinds of logical application servers and 5GMSGS server is needed to be defined.

## 5.10	Key Issue 10: broadcast area configuration

### 5.10.1	Description

The service requirement of broadcast area configuration is specified in requirement [R-5.5.2-002] of 3GPP TS 22.262 [2].

[R-5.5.2-002] The MSGin5G Service shall support an authorized application server or UE to send a broadcast message to all the UEs within a specific area which is configured according to application policy.

and 5.5.1 of 3GPP TS 22.262 [2]

The receivers of broadcast messages can be all UEs within a cell or multiple cells. The broadcast areas can be configured according to the policy of application.

The requirement applies to broadcast message scenarios.

The following conclusion can be made based on the requirements above:

-	broadcast area configuration should be specified in MSGin5G application layer.

Issues include:

-	Whether the existing broadcast area configuration, i.e. cell broadcast service specified in 3GPP TS 23.041 [6] supports the broadcast area configuration requirement specified in 3GPP TS 22.262 [2]

-	What and how broadcast areas configuration related information can be used in the application layer

### 5.10.2	Identified Gaps

The following gaps have been identified to fulfil the identifier requirements:

-	What is broadcast area configuration information used for MSGin5G broadcast message.

-	How broadcast areas configuration related information can be used in the application layer.

-	The broadcast area configuration interworking in the MSGin5G gateway are needed to be defined.

## 5.11	Key Issue 11: mobility management of MSGin5G related information

### 5.11.1	Description

The service requirement of mobility management of the UE is specified in requirement [R-5.1.2-008] of 3GPP TS 22.262 [2]. It applies to all message scenarios.

[R-5.1.2-008] The MSGin5G Service shall support the mobility of a UE (i.e. the UE can still send/receive messages when it changes the location of network access).

Issues include:

-	What mobility management of MSGin5G related information are required to be supported in MSGin5G service application layer.

-	Whether and how additional service APIs are required to be supported in MSGin5G service application layer,

### 5.11.2	Identified Gaps

The following gaps have been identified to fulfil the identifier requirements:

-	The mobility management of MSGin5G related information which fulfils the service requirement is needed to be defined.in MSGin5G service application layer.

-	Specify an MSGin5G mobility management on top of 5GS if needed.

-	How to manage the mobility of non-3GPP message UE and the related procedure are needed to be studied.

## 5.12	Key Issue 12: End-to-end latency

### 5.12.1	Description

In clause 5.1 of 3GPP TS 22.262 [2], the requirement of end-to-end latency is specified as below:

[R-5.1.2-001] The MSGin5G Service shall support UE sending and receiving a text or data message with end-to-end latency less than [500] ms.

NOTE 1:	Initial connection activation latencies may be longer depending on receiving UE power saving states, paging, etc.

The requirement [R-5.1.2-001] is specified in the general clause of service requirements so it will apply to all of the four message scenarios. A similar requirement [R-5.5.2-001] listed below is also listed in clause 5.5 of 3GPP TS 22.262 [2].

[R-5.5.2-001] The MSGin5G Service shall support broadcasting a text or data message with end-to-end latency less than [500] ms.

A further requirement for the Application-to-point message scenario listed below is also specified in 3GPP TS 22.262 [2]:

[R-5.3.2-003] The MSGin5G Service shall support Application Originated Mobile Terminated messaging service with max latency of 10 seconds while maintaining battery life of at least 3 months for small data traffic once every hour and typical sized IOT battery [200-500mAh].

This requirement applies to the AOMT messaging service, so [R-5.1.2-001] still applies to the MOAT messaging service.

Issues include:

-	The main factors affecting the end-to-end latency in each message scenario are needed to be studied.

-	How to ensure that the end-to-end latency in point-to-point message scenario is less than [500] ms.

-	How to ensure that the end-to-end latency in AOMT messaging service is less than [10] seconds.

-	How to ensure that the end-to-end latency in MOAT messaging service is less than [500] ms.

-	How to ensure that the end-to-end latency in Group message scenario is less than [500] ms.

-	How to ensure that the end-to-end latency in Broadcast message scenario is less than [500] ms

### 5.12.2	Identified Gaps

The following gaps have been identified to fulfil the identified requirements:

-	Whether some procedures in 5G system can be used to limit the end-to-end messaging latency is needed to be studied. The procedures may be different in each message scenario.

-	If new procedures are used, the potential impact on 5G system is needed to be studied.

-	Specify some MSGin5G specific procedures to reduce the end-to-end messaging latency on top of 5GS if needed.

## 5.13	Key Issue 13: Message store and forward

### 5.13.1	Description

In many MSGin5G Service use cases, a targeted UE may not be immediately available to receive a message (e.g. UE may be disconnected or powered-off).  In such cases, the MSGin5G Service may not be able to deliver the message successfully to a targeted UE.

To satisfy these types of use cases, a requirement for  supporting Message store and forward capabilities is specified in [R-5.1.2-005] of 3GPP TS 22.262 [2] below:

[R-5.1.2-005]	The MSGin5G Service shall support storage of a message if a UE is unavailable (disconnected or power off) for future delivery once the UE becomes available.

The requirement [R-5.1.2-005] is specified in the general clause so it will apply to all of the four message scenarios.

Store-and-forwarding of messages by the MSGin5G Service enables the storing (i.e. buffering) of messages when targeted UEs are unavailable and the forwarding of stored messages to targeted UEs once they become available. By supporting store-and-forwarding of messages, the MSGin5G Service can significantly improve the overall performance and efficiency of the system.

Issues include:

-	How the MSGin5G Service can determine that theUE (including 5GMSGS UE, legacy-3GPP UE and non-3GPP UE) is unavailable e.g. based on the application layer registration status,network  connection status API provided by the lower layer, etc.

-	How to determine that store and forward is needed.

-	What is the relationship between store and forward mechanism and other application feature, e.g. network triggering specified in [R-5.1.2-006] of 3GPP TS 22.262 [2]. If the UE is unavailable, whether the MSGin5G service needs to notify the UE that there are messages waiting for it to pick up.

-	How to determine that the UE becomes available.

-	If a message is stored by the MSGin5G Service, how long can the message be stored waiting for a UE to become available.

### 5.13.2	Identified Gaps

The following gaps have been identified to fulfil the identified requirements:

-	The procedure to identify if the UE (including 5GMSGS UE, legacy-3GPP UE and non-3GPP UE) is available or unavailable to receive MSGin5G message needs to be studied.

-	Study how to identify that store and forward mechanism needs to be used.

## 5.14	Key Issue 14: Message Segmentation and Reassembly

### 5.14.1	Description

3GPP TS 22.262[2] defines the following requirement in the general clause, therefore being applicable to all of the four message scenarios:

[R-5.1.2-002] The MSGin5G Service shall support variable size of payload of text and data with a maximum of 2048 bytes and support segmented transmission if the content is larger than the maximum payload length of a message.

### 5.14.2	Identified Gaps

The following aspects have been identified for study:

-	Weather a fixed-length segment size (e.g. 2048 octets) fulfils the segmentation service requirements.

Editor's Note: The size of the segmentation packet meeting 3GPP TS 22.262 [2] requirements is FFS.

-	Whether there are differences in handling between the four message scenarios.

-	The relationship between message segmentation and other service requirements, e.g. store and forward (specified in [R-5.1.2-005] of 3GPP TS 22.262 [2]).

The following capability gaps have been identified:

-	The capability for the transmitting MSGin5G Service endpoint to detect when the size of a MSGin5G message exceeds the maximum packet size.

-	The capability for the transmitting MSGin5G Service endpoint to segment and deliver a MSGin5G message that exceeds the maximum packet size into a set of segmented messages that individually do not exceed the maximum packet size.

-	The capability for the receiving MSGin5G Service endpoint to reassemble a set of segmented messages received via individual packets into a single MSGin5G message.

-	The capability for the receiving MSGin5G Service endpoint to indicate successful reception of all the segmented messages.

-	The capability for the receiving MSGin5G Service endpoint to detect the lost segmented messages and request the transmitting MSGin5G Service endpoint to retransmit.

-	The capability for the transmitting MSGin5G Service endpoint to retransmit the segmented messages that are lost.

## 5.15	Key Issue 15: Message delivery from MSGin5G Server

### 5.15.1	Description

The key issue addresses message delivery from the MSGin5G Server to an end-point or to multiple end-points.

It is assumed that the MSGin5G Server is the only entity in the MSGin5G Service that is aware how messages need to be forwarded towards the various UEs that participate in the service and that only the MSGin5G Server has access to a broadcast function, an SMS delivery function in the 5GC, and to a MSGin5G Gateway.

### 5.15.2	Identified Gaps

The following gaps are identified:

- The MSGin5G Server needs to be able to forward messages to the requested end-point, based on UE types (determined by the Service ID).

- The MSGin5G Server needs to be able to forward messages to a Group of UEs. The MSGin5G Server needs to forward the message to each of the Group members. Further details on the group message are described in Key Issue 7.

- The MSGin5G Server needs to be able to forward messages to the broadcast function in the 5GC, so the message will be broadcast in a service area.

- The MSGin5G Server needs to be able to forward messages to the Application Server. The Application Server may reside outside the 3GPP domain and may be connected through the NEF to 5GC.

- The MSGin5G Server needs to be able to forward messages to the MSGin5G Gateway for UEs that support a non-3GPP transport mechanism. Further details on message transfer to a MSGin5G Gateway are described in Key Issue 6.

## 5.16	Key Issue 16: Charging

### 5.16.1	Description

The key issue addresses charging aspects as listed in requirements [R-6.2-001] and [R-6.2-002] in TS 22.262 [2].

All message transfer passes through the MSGin5G Server and it can therefore be assumed that generating charging records will be done in the MSGin5G Server.

### 5.16.2	Identified Gaps

The following gaps are identified:

- What information is needed by the MSGin5G Server to generate charging records according to operator or application service provider policy?

- How operator and application service provider charging policies can be configured and modified at the MSGin5G Server.

- Is the 3GPP charging management framework as specified in TS 32.290 [x] applicable for these charging policies?

- What information is needed by the interworking gateway to generate charging records according to operator or application service provider policy?

## 5.17	Key Issue 17: Interconnection

### 5.17.1	Description

The requirement of MSGin5G Interconnection listed below is specified in [R-9.2-001] and [R-9.2-002] of 3GPP TS 22.262 [2].

Issues include:

- If MSGin5G services are provided by different PLMNs, how to exchange the MSGin5G service capabilities between the PLMNs.

- Whether all four message scenarios (as specified in clause 4) can be interconnected between difference PLMNs.

- If the MSGin5G service capabilities between different PLMNs are different, how to interconnect MSGin5G services between different PLMNs.

- Whether procedures for group management or configuration management for interconnection among different PLMNs are needed.

### 5.17.2	Identified Gaps

The following gaps have been identified to fulfil the identified requirements:

- The information of service capabilities needed to be exchange between different PLMNs and the related exchanging procedures are needed to be studied.

- When at least one party of the message exchang (e.g. to identify UE is in different PLMN, to register UE from different PLMN, to send and receive message from different PLMN, etc.) belongs to a different PLMN, the procedures of the message exchanging are needed to be studied. This gap applies to all of the four message scenarios.

## 5.18	Key Issue 18: Roaming

### 5.18.1	Description

The requirement of MSGin5G Roaming listed below is specified in clause 8.2 of 3GPP TS 22.262 [2].

[R-8.2-001] The MSGin5G Service shall support a roaming UE sending and receiving messages.

There are multiple options for MSGin5G roaming when UE moves from home PLMN to visited PLMN:

- The MSGin5G service is always provided by the home PLMN.

- The MSGin5G service is provided by the visited PLMN if MSGin5G service is supported by the visited PLMN, otherwise the MSGin5G service is provided by the home PLMN.

Different option may have different application architecture requirements.

Issues include:

- How to provide MSGin5G service when the UE is roaming, i.e. which option listed above is suitable to be chosen.

- Whether network architectures of different roaming scenario (i.e. local breakout and home routed) affect MSGin5G services.

- Whether MSGin5G Roaming is needed to be supported by the network layer. How the network layer supports MSGin5G Roaming.

- Whether some requirements, e.g. end-to-end delay specified in [R-5.1.2-001] of 3GPP TS 22.262 [2] and network triggering specified in [R-5.1.2-006] of 3GPP TS 22.262 [2], can be fulfilled in roaming scenario.

### 5.18.2	Identified Gaps

The following gaps have been identified to fulfil the identified requirements:

- Application layer architecture to support roaming of 5GMSGS UEs and the related procedures for roaming UEs are needed to be studied.

- If MSGin5G Roaming is needed to be supported by the network layer, the potential impacts to on 5G system are needed to be studied.

## 5.19	Key Issue 19: Efficient resource usage of the control plane and user plane

### 5.19.1	Description

The typical IoT device communication is sending and receiving small data which can be delivered just in a message. The characteristics of Massive Internet of Things (MIoT) devices including high density connection, flexible mobility, saving power, limited computing capability, bulk of devices, and traffic pattern of short burst of small data brings various new demands on message communication, e.g. light weight message communication for provision and monitoring, ultra low latency and high reliability message communication for remote control, and extremely high resource efficiency for large scale connections. The MSGin5G Service is basically designed and optimized for massive IoT device communication including thing-to-thing communication and person-to-thing communication.

Following requirement is described in 3GPP TS 22.262 [2].

Considering the massive connections of IoT devices and high throughput of message communication between devices or between devices and application servers, the MSGin5G Service needs to be in a resource efficient manner to optimize the resource usage of the both control plane and user plane.

Following generic requirement is also specified in 3GPP TS 22.262 [2].

The IoT devices usually have limitation in computation and storage, and are powered by batteries or small solar photovoltaic equipment, so the message communications need to be light weight and well scheduled in order to save power and data traffic consumption in the device.

The following conclusion can be made based on the requirements above:

-	There is extremely high number of message exchange between devices or between devices and application servers.

-	The typical IoT device communication is sending and receiving small data which can be delivered just in a message.

-	The requirement applies to all type of message communication models.

-	The communication needs to be well scheduled in order to save power and data traffic consumption in the device.

In principle, the MSGin5G Service needs to enable optimization of the resource usage for both the control plane and user plane in a resource efficient manner.

Issues include:

-	How to make MSGin5G communication resource efficient to optimize the resource usage of both the control plane and user plane?

-	What scheduling policy needs to be followed in order to make sure that application server is not overloaded with messages at particular time?

-	How MSGin5G service can support distributing the scheduling policy to application client?

-	How to efficiently use resources for sending and receiving of typical small data?

### 5.19.2	Identified Gaps

The following gaps have been identified to fulfil the efficient resource usage of the control plane and user plane requirements:

-	Mechanisms to support small data message exchange in a resource efficient manner.

-	Determining scheduling policy and making it available to 5GMSGS client.

-	The behavior on 5GMSGS client for using resource efficiently and the related procedures are needed to be studied.

# 6	Solutions

## 6.1	Solution 1: Payload size of 2048 octets in Broadcast

### 6.1.1	Description

3GPP TS 23.041 [6] specifies in subclause 9.3.51 the maximum payload for the CB Data element to be 1230 octets. The payload consists of N pages of 82 octets each and N is equal to 15 or less.

By allowing N to be 25 or less, the maximum payload can be increased to 2050 octets.

### 6.1.2	Impacts on existing nodes and functionality

There is no backwards compatibility with this solution. The CBCF, the gNodeBs and the UEs will need to support a maximum of 25 pages of CB Data instead of the 15 as they do now.

### 6.1.3	Solution evaluation

Editor's Note: Evaluation depends on outcome of RAN2 guidance.

## 6.2	Solution 2: Registering 5GMSGS Client to MSGin5G Server

### 6.2.1	Description

#### 6.2.1.1	General

This solution describes how the MSGin5G server identifies a UE and the Application Clients on the UE that interact with the MSGin5G service. This solution addresses the following open issue/gap under Key Issue #3.

-	How to uniquely identify a UE in MSGin5G service, the UE may include5GMSGS UEs, Legacy 3GPP UEs and non-3GPP UEs.

-	If the identifier(s) of UE is assigned by MSGin5G server, the procedure of generating, assigning and managing the identifier(s) are needed to be defined.

The solution introduces a registration procedure for a 5GMSGS Client to register with a MSGin5G Server.  The procedure is used to create a new, or update an existing, registration. The procedure is also used to provide the 5GMSGS Client with a unique identifier that is assigned by the MSGin5G Server. During registration, the 5GMSGS Client provides profile/availability information for the 5GMSGS Client and the Application Clients that are serviced by the 5GMSGS Client to the MSGin5G Server. The profile/availability information includes contact information such as UE Identifier(s) and port number(s) which the 5GMSGS Client and the Application Clients listen on for incoming MSGin5G messages, supported MSGin5G capabilities (e.g. MOMT, AOMT, MOAT, Group, Broadcast) and MSGin5G service requirements (e.g. required time windows of service, message latency and data rates). The MSGin5G Server validates the profile information to determine whether the MSGin5G requirements specified in the profiles can be fulfilled. The MSGin5G Server also stores the profile information such that this information can be later used by the MSGin5G Server to deliver MSGin5G messages to/from the 5GMSGS Client and Application Clients.

The solution also introduces a 5GMSGS Client de-registration procedure. This procedure is used by a 5GMSGS Client to de-register from an MSGin5G Server.

#### 6.2.1.2	5GMSGS Client Registration

The signalling flow for 5GMSGS Client registration is illustrated in figure 6.2.1.2-1. The solution assumes that the 5GMSGS Client is responsible for triggering the registration to the MSGin5G Server.

NOTE 1:	The trigger for registering is based on application service logic and is out of scope of this specification.

Pre-conditions:

1.	The UE on which the 5GMSGS Client resides is connected to an access network that provides connectivity to the MSGin5G Server.

2.	The 5GMSGS Client has been configured with the MSGin5G Server Identity.

3.	The 5GMSGS Client has been configured with the address (e.g. URI) of the MSGin5G Server.

4.	Both the 5GMSGS Client and MSGin5G Server have been configured with the necessary credentials to enable authenticating one another.

NOTE 2:	The structure and definition of the credentials required by the 5GMSGS Client and MSGin5G Server to authenticate are left for SA3 to define.

Figure 6.2.1.2-1: MSGin5G Client registration

1.	The 5GMSGS Client sends a 5GMSGS Client registration request (5GMSGS Client Profile) to the MSGin5G Server. The request includes security credentials required for the 5GMSGS Client to register to the MSGin5G Server.  The request also includes a 5GMSGS Client Profile for the 5GMSGS Client initiating the registration request.  The 5GMSGS Client Profile includes the information listed in Table 6.2.1.2-1.

Table 6.2.1.2-1: 5GMSGS Client Profile

| Information element | Status | Description |
| --- | --- | --- |
| UE ID | M | Identity of the UE hosting the 5GMSGS Client (e.g. the External Identifier defined in TS 23.682 [x], or an MSISDN) |
| 5GMSGS Client Ports | M | List of port numbers that the 5GMSGS Client listens on for messages (e.g. device triggers) from the MSGin5G Server and that target the 5GMSGS Client. Also included with each port number is an associated protocol (e.g. SMS, NIDD, etc.). |
| 5GMSGS Client ID | O | 5GMSGS Client identifier assigned to the 5GMSGS Client by the MSGin5G Server upon initial registration. |
| 5GMSGS Client Capabilities | O | A list of the MSGin5G capabilities supported by the 5GMSGS Client (e.g. MOMT, AOMT, MOAT, Group, Broadcast) |

2.	Upon receiving the request from the 5GMSGS Client, the MSGin5G Server validates the registration request and verifies the security credentials. The MSGin5G Server also checks if a 5GMSGS Client ID is present within the 5GMSGS Client Profile of the request to determine if the request is a new registration or an update to an existing registration. A registration update may be used to send a new or updated 5GMSGS Client Profile to the MSGin5G Server. The MSGin5G Server also checks if the 5GMSGS Client Capabilities are compatible with its own MSGin5G Server Capabilities.

3.	The MSGin5G Server sends a 5GMSGS Client registration response to the 5GMSGS Client. If the registration is a new registration, the MSGin5G Server assigns a unique 5GMSGS Client ID to the 5GMSGS Client and returns it in the response. The 5GMSGS Client stores the identifier and uses it in all future MSGin5G communication with the MSGin5G Server. If the MSGin5G Server is not able to fulfil, or provide, the MSGin5G services that are indicated within the 5GMSGS Client Profile, then the MSGin5G Server will indicate this in the 5GMSGS Client registration response. If the registration is successful, the MSGin5G Server stores the profile information such that this information can be later used by the MSGin5G Server to deliver MSGin5G messages to/from the 5GMSGS Client and Application Clients.

#### 6.2.1.3	5GMSGS Client De-Registration

By de-registering, the 5GMSGS Client informs the MSGin5G Server that it is no longer available to send or receive MSGin5G messages to or from the MSGin5G Server and wishes to terminate its association with the MSGin5G Server.

The procedure assumes that the 5GMSGS Client is responsible for triggering the de-registration from the MSGin5G Server.

NOTE:	The trigger condition for de-registering is based on application service logic and is out of scope of this specification.

The signalling flow for 5GMSGS Client de-registration is illustrated in figure 6.2.1.3-1.

Pre-conditions:

1.	The 5GMSGS Client is registered to the MSGin5G Server.

Figure 6.2.1.3-1: 5GMSGS Client De-registration

1.	The 5GMSGS Client determines that de-registration from the MSGin5G Server is needed (e.g. the UE powering down).

2.	The 5GMSGS Client sends a De-registration Request to the MSGin5G Server that includes the 5GMSGS Client ID of the 5GMSGS Client.

3.	The MSGin5G Server deletes any applicable 5GMSGS Client Profile information that it has stored and replies to the 5GMSGS Client with a De-registration Response.

### 6.2.2	Impacts on existing nodes and functionality

There is no backwards compatibility with this solution.

### 6.2.3	Solution evaluation

This solution allows a 5GMSGS Client to register to a MSGin5G Server. The solution enables a MSGin5G Server to assign a unique identity to a 5GMSGS Client and to validate whether MSGin5G capabilities of the 5GMSGS Client are compatible with MSGin5G capabilities of the MSGin5G Server. The solution enables a 5GSMSG Client to share contact information (e.g. UE identifiers and port numbers) with a MSGin5G Server.

## 6.3	Solution 3: MSGin5G Triggering

### 6.3.1	Description

#### 6.3.1.1	General

This solution addresses the identified gap under Key Issue #4 regarding specification of a MSGin5G trigger procedure.

The solution introduces a procedure that allows a MSGin5G Server to initiate a MSGin5G trigger to a UE that targets either a 5GMSGS Client or Application Client on the UE.  Within the MSGin5G trigger request, a MSGin5G trigger payload is defined that includes informational elements such as the purpose of the trigger so that trigger actions can be performed by the targeted 5GMSGS Client or Application Client on the UE. The procedure is agnostic to access network technologies and can be used over top of 3GPP as well as non-3GPP access networks.

#### 6.3.1.2	MSGin5G Triggering

Figure 6.3.1.2-1 shows the MSGin5G triggering procedure.

Pre-conditions:

1.	A UE hosts a 5GMSGS Client and/or Application Specific Client which are supported by the MSGin5G service.

2.	The 5GMSGS Client registers with the MSGin5G Server and shares UE contact information (e.g. UE access network identifier and port number(s)). Alternatively, the MSGin5G Server may be provisioned with the contact information (e.g. for UEs that do not host a 5GMSGS Client and cannot register to a MSGin5G Server).

3.	At a later time, the UE becomes unreachable by the MSGin5G Server.

Figure 6.3.1.2-1: 5GMSGS Triggering Procedure

1.	An Application Server initiates a request to a MSGin5G Server to send a MSGin5G message to a targeted 5GMSGS client or Application Client on a UE.

2.	The MSGin5G Server determines that the UE is not reachable at this time and sends a trigger request to the UE. For example, if a MSGin5G Server detects that the UE does not have an access network connection and the MSGin5G Server needs to send a MSGin5G message to the UE, then the MSGin5G Server sends a MSGin5G trigger to the UE to have it establish an access network connection.  This trigger is initiated as a result of a MSGin5G request received from an Application Server that targets an Application Client on a UE. Alternatively, a MSGin5G Server initiates a trigger to a UE independent of receiving a MSGin5G request from an Application Server.

If the UE is reachable then the trigger request is not required and the MSGin5G Server sends the MSGin5G message to the UE as requested by the Application Server.

3.	The MSGin5G Server determines the access network (e.g. 3GPP, etc.) and the access network message delivery method (e.g. SMS, NIDD, etc.) to deliver the trigger request to the targeted UE. To make this determination, the MSGin5G Server can rely on information provided to the MSGin5G Server by the 5GMSGS Client of the targeted UE during the 5GMSGS Client registration procedure. Alternatively, the MSGin5G Server can rely on information for the targeted UE that is provisioned into the MSGin5G Server.

4.	The MSGin5G Server sends a MSGin5G trigger request to the targeted UE which includes the information in Table 6.3.1.2-1. The MSGin5G Server embeds the MSGin5G trigger request information within an access network trigger request (e.g. an SMS trigger). For example, the MSGin5G Server configures the destination address of the access network trigger request with the UE Identifier of the targeted UE. The MSGin5G Server configures the destination port of the access network trigger request with the 5GMSGS Client Port if the UE supports a 5GMSGS Client or an Application Client Port if the UE does not support a 5GMSGS Client. The MSGin5G server embeds the Trigger Purpose within the payload of the access network trigger request.

NOTE 1: The details of how a MSGin5G Server maps the information within a MSGin5G trigger request to an access network trigger request is dependent on the type of access network and is out of scope of this study.

Table 6.3.1.2-1: MSGin5G Trigger Request

| Information element | Status | Description |
| --- | --- | --- |
| UE Identifier | M | The identifier of the UE (e.g. 3GPP External Identifier, non-3GPP device identifier, IMSI, etc.) |
| Client Port | O | The port number that the 5GMSGS Client or Application Client is listening on to receive trigger requests. |
| Trigger Purpose | M | Specifies the purpose of the trigger request. - For a 5GMSGS Client or Application Client on the UE to establish an access network connection - For a 5GMSGS Client on the UE to register to the MSGin5G Server - For an Application Client on the UE to be available for a period of time to receive a request from an Application Server |
| Trigger Info | M | Access Network specific information used to deliver the trigger payload information, e.g. reference number, validity period etc. |

5.	The access network delivers the trigger request to the targeted UE.

NOTE 2: The access network trigger delivery procedures are out of scope of this study.

6.	The MSGin5G Server receives a response from the access network indicating the success or failure status of the request. The type of status indication provided depends on the capabilities of the access network mechanism used.

7.	The targeted 5GMSGS Client and/or Application Client on the UE listens on its respective port number and receives the access network trigger request.  The targeted 5GMSGS Client and/or Application Client parses the payload of the access network trigger request which contains the Trigger Purpose.

Based on the Trigger Purpose, the targeted 5GMSGS Client or Application Client performs the corresponding actions (e.g. establish access network connectivity, update Application Server with Application Client's contact information, etc).

### 6.3.2	Impacts on existing nodes and functionality

This solution leverages existing access network triggering functionality and defines a MSGin5G trigger procedure that overlays on top of them, hence there is no backwards compatibility issues or impact with this solution.

### 6.3.3	Solution evaluation

The solution defines a MSGin5G trigger procedure that is agnostic to access network technologies and can be used over top of 3GPP as well as non-3GPP access networks should be defined.  This enables the MSGin5G Service to trigger 3GPP as well as non-3GPP devices if/when required.

## 6.4	Solution 4: Group management using SEAL

### 6.4.1	General

The following solution corresponds to the key issue #8 describing the group management using the procedures in SEAL TS 23.434 [9].

Editor's note:	The solution alignment based on inclusion of SEAL in the architecture figure is FFS.

### 6.4.2	Solution description for establishing a group

The solution in SEAL TS 23.434 [9] subclause 10.3.3 is used for group creation in MSGin5G Service. The 5GMSGS Client (or the Application Server) acts as the Group management client and the MSGin5G Server acts as the Group management server as specified in 3GPP TS 23.434 [9].

### 6.4.3	Solution description for adding UEs to the group or removing UEs from the group

The solution in SEAL TS 23.434[9] subclause 10.3.5.2 is used for adding UEs to the group or removing UEs from the group in MSGin5G Service. The 5GMSGS Client (or the Application Server) acts as the Group management client and the MSGin5G Server acts as the Group management server as specified in 3GPP TS 23.434 [9].

### 6.4.4	Solution description for configuration of a maximum number of members in a group

The solution in SEAL TS 23.434[9] subclause 10.3.6.1 is used for configuration of a maximum number of members in a group in MSGin5G Service. The 5GMSGS Client (or the Application Server) acts as the Group management client and the MSGin5G Server acts as the Group management server as specified in 3GPP TS 23.434 [9].

### 6.4.5	Solution evaluation

Editor's Note: Solution evaluation is FFS.

## 6.5	Solution 5: 5GMSGS Group messaging

### 6.5.1	Description

#### 6.5.1.1	General

This solution addresses the identified gap under Key Issue #7 regarding 5GMSGS group messaging.

This solution introduces a group messaging procedure for 5GMSGS Client and MSGin5G server to send and receive Group message after a group is created. In this procedure, the group creation and membership management are handled by SEAL specified in 3GPP TS 23.434 [9].

Editor's Note 1: The procedure of Group message without using SEAL is FFS.

Editor's Note 2:	How the message will be routed to non-IMS UE from MSGin5G server is FFS.

Editor's Note 3:	Definition of Service ID and it's usage for MSGin5G Service is FFS.

#### 6.5.1.2	5GMSGS Group messaging

Figure 6.5.1.2-1 shows the 5GMSGS Group messaging procedure in which 5GMSGS client (both IMS and non-IMS UE) sends a message to a group.

Editor's note:	Solution for the group hosted on the legacy side where 5GMSGS client is one of the group member is FFS.

Pre-conditions:

1)	A 5GMSGS Group is created by following group management SEAL service procedures as specified in 3GPP TS 23.434 [9].

2)	All participants in the 5GMSGS Group may get the Group information i.e. a list of group members in the Group information query response as specified in 3GPP TS 23.434 [9].

3)	The group consists of different types of UEs i.e. 5GMSGS UE, Legacy 3GPP UE and Non-3GPP UE.

Figure 6.5.1.2-1: Group messaging in MSGin5G service

1)	The 5GMSGS client 1 sends a message to a group as specified in clause 6.5.1.3.

2)	Upon receiving the request to send the group message, the MSGin5G server may send the message to the application server based on service ID present in the received Group Message Request (e.g. to log application specific message or for analytics). Otherwise go to step 4.

3)	The application server initiates to send message to all group members and sends the Group Message Request to the MSGin5G server.

Editor's note:	Consideration of other cases (e.g. failure) in step 2 and 3 is FFS.

4)	The MSGin5G server sends the message to all participants of the group based on UE types (determined by service ID) as follows:

a)	if the target UE supports 5GMSGS client, the MSGin5G server sends the message as specified in clause 6.5.1.4.

b)	if the target UE supports Legacy 3GPP message client, the MSGin5G server sends the message as specified in clause 6.5.1.5.

c)	if the target UE supports Non-3GPP message client, the MSGin5G server sends the message as specified in clause 6.5.1.6.

NOTE:	Steps 4-a), 4-b) and 4-c) can happen in parallel and in any order.

5)	Upon receiving the group message, if delivery status is requested and if supported by target message client, the 5GMSGS client or Legacy 3GPP message client or Non-3GPP message client sends the delivery status message to originator 5GMSGS client 1.

#### 6.5.1.3	5GMSGS client sends message to group

Figure 6.5.1.3-1 shows the 5GMSGS Group messaging procedure which 5GMSGS client initiates sending a message to a group.

Pre-conditions:

1)	A UE hosts a 5GMSGS client and an Application Client.

2)	The non-IMS 5GMSGS client registers with the MSGin5G Server.

3)	Information to make selection of group to send message is available at the Application Client.

Figure 6.5.1.3-1: 5GMSGS client sends message to a group

1)	An Application Client on a UE requests to an 5GMSGS client to send a message to a group.

2)	The 5GMSGS client sends the Group Message Request to the MSGin5G server. The information elements defined in Table 6.5.1.3-1 are included in the request.

Table 6.5.1.3-1: Group Message Request (5GMSGS client to MSGin5G Server)

| Information element | Status | Description |
| --- | --- | --- |
| Originator 5GMSGS client service ID | M | The identity of the 5GMSGS client sending the message. |
| Group ID | M | The identifier of the target 5GMSGS Group. |
| Message ID | M | Unique identifier of this message |
| Disposition Type | O | Indicates the disposition type expected from the receiver (i.e., delivered) |
| Application ID | O | Identifies the application for which the payload is intended. Generally same as application ID of the application which initiated this message. |
| Payload | M | Payload of the message |
| Priority Type | O | Application priority level requested for this message. |

3)	MSGin5G server checks whether the 5GMSGS client 1 is authorized to send Group Message Request. If authorized, the MSGin5G server resolves the 5GMSGS Group ID to determine the members of that group and determines the registration status of non-IMS UEs, based on the information from the group management server. The MSGin5G server validates the request based on the policy e.g. to limit certain types of message or content to certain members due to location or user privilege.

Editor's note:	The usage of priority type and validation of request based on policy (i.e. based on location or user previleges) at MSGin5G server are FFS.

4)	If 5GMSGS client 1 is not authorized to send Group Message Request or the group message request is not valid, the MSGin5G server sends Group Message Reject message to the 5GMSGS client 1. The information elements defined in Table 6.5.1.3-2 are included in the request.

Table 6.5.1.3-2: Group Message Reject message

| Information element | Status | Description |
| --- | --- | --- |
| Originator 5GMSGS client service ID | M | The identity of the 5GMSGS client who initiated the Group Message Request |
| Group ID | M | The identifier of the 5GMSGS Group. |
| Message ID | M | Identifier contained in the Group Message Request |
| Reject Code | O | The code for reject reason |

#### 6.5.1.4	5GMSGS client receives message from group

Figure 6.5.1.4-1 shows the procedure for 5GMSGS client to receive message from group.

Pre-conditions:

1)	A UE hosts a 5GMSGS client and an Application Client

2)	The non-IMS 5GMSGS client registers with the MSGin5G server.

3)	The MSGin5G server is aware of the UE type (determined by service ID) which is 5GMSGS client.

4)	The MSGin5G server has received the message to send to target UE.

Figure 6.5.1.4-1: 5GMSGS client receives a message from the group

1)	The MSGin5G server sends the Group Message Request to the 5GMSGS client. The information elements defined in Table 6.5.1.4-1 are included in the request.

Table 6.5.1.4-1: Group Message Request (MSGin5G server to 5GMSGS client)

| Information element | Status | Description |
| --- | --- | --- |
| Originator 5GMSGS client service ID | M | The identity of the 5GMSGS client who initiated the message. |
| Group ID | M | The identifier of the 5GMSGS Group to which the message is sent |
| Recipient ID | M | The identity of the 5GMSGS client service ID of the receiver UE. |
| Message ID | M | Unique identifier of this message |
| Disposition Type | O | Indicates the disposition type expected from the receiver (i.e. delivered) as specified in Group Message Request received from originating 5GMSGS client |
| Application ID | O | Identifies the application for which the payload is intended as specified in Group Message Request received from originating 5GMSGS client |
| Payload | M | Payload of the message |
| Priority Type | O | Application priority level requested for this message as specified in Group Message Request received from originating 5GMSGS client. |

Editor's note:	The usage of priority type and validation of request based on policy (i.e. based on location or user previleges) at MSGin5G server are FFS.

2)	The 5GMSGS client delivers the contents of the MSGin5G message to the targeted Application Client.

#### 6.5.1.5	Legacy 3GPP message client receives message from group

Figure 6.5.1.5-1 shows the procedure for Legacy 3GPP message client to receive message from group.

Pre-conditions:

1)	Based on service ID, an application server has received the group message from the originator.

2)	The MSGin5G server is aware of the UE type (determined by service ID) which is Legacy 3GPP message client.

3)	The MSGin5G server has received the message to send to target UE.

Figure 6.5.1.5-1: Legacy 3GPP message client receives a message from the group

1)	The MSGin5G server translates the 5GMSGS Group message to legacy 3GPP message (e.g. SMS).

Editor's Note:	Step 1 needs to be aligned with SA1 requirement related to translation of MSGin5G message to legacy 3GPP message is revised.

2) The MSGin5G server sends legacy 3GPP message (e.g. SMS) to 3GPP Message Client.

NOTE:	The procedure to send SMS from MSGin5G server to legacy 3GPP UE is FFS.

3) The 3GPP Message Client delivers the contents of the legacy 3GPP message (e.g. SMS) to the targeted Application Client.

#### 6.5.1.6	Non-3GPP message client receives message from group

Figure 6.5.1.6-1 shows the procedure for Non-3GPP message client to receive message from group.

Pre-conditions:

1)	The MSGin5G server is aware of the UE type (determined by service ID) which is Non-3GPP message client.

2)	The MSGin5G server has received the message to send to target UE.

Figure 6.5.1.6-1: Non-3GPP message client receives a message from the group l

1)	The MSGin5G server sends the Group Message Request to the 5GMSGS Gateway. The information elements defined in Table 6.5.1.4-1 are included in the request.

2)	The 5GMSGS Gateway translates the MSGin5G message to Non-3GPP message.

3)	The 5GMSGS Gateway sends message to Non-3GPP Message Client.

NOTE:	The procedure to translate MSGin5G message to Non-3GPP message and to send Non-3GPP message from 5GMSGS Gateway to Non-3GPP Message Client are out of scope of 3GPP.

### 6.5.2	Impacts on existing nodes and functionality

SEAL specified in 3GPP TS 23.434 is needed to be introduced if this solution is used for 5GMSGS Group messaging. Before Group message exchange, a 5GMSGS Group is needed to be created by using SEAL. The identifier of the group created by SEAL is needed to be assigned according to the rules of MSGin5G services. Enhancements to SEAL for supporting group management in case of a non-IMS UE is required.

### 6.5.3	Solution evaluation

This procedure can be used by 5GMSGS Client, Legacy-3GPP Client, 5GMSGS Gateway Function and MSGin5G Server to send and receive group message. This procedure can simplify the functional requirements for the 5GMSGS Client and MSGin5G Server.

## 6.6	Solution 6: Charging

### 6.6.1	Description

Editor's Note: Any solution needs to distinguish between UE charging and vertical/tenant charging and all charging solutions for MSGin5G services should be specified in SA5 which should be involved at some stage.

## 6.7	Solution 7: MSGin5G Server message broadcast

### 6.7.1	Description

#### 6.7.1.1	General

All MSGin5G message traffic passes through the MSGin5G Server, including broadcast messages. Broadcast messages originate from an Application Client on a UE or from the Application Server and need to be broadcast in a pre-configured service area.

The only broadcast service that has been specified for 5GS Release-16 is the Cell Broadcast Service as specified in 3GPP TS 23.041 [6]. If the modification in 3GPP TS 23.041 [6] to extend the maximum payload, as proposed in Solution 1, is implemented then Cell Broadcast can be used for the broadcast message. If the modification is not implemented, then the broadcast message may need to be segmented.

#### 6.7.1.2	MSGin5G message broadcast

Figure 6.7.1.2-1 shows the 5GMSGS broadcast message procedure which message is sent by an Application Server. The broadcast message can also be initiated from an Application Client. The payload shall be received by Application Clients located in the service area.

Pre-conditions:

1.	The service areas are pre-configured in the MSGin5G Server: Each service area is identified by a Service Area ID. The polygon that defines the service area is stored in the CBCF and can be addressed by its Service Area ID.

Figure 6.7.1.2-1: Broadcast message procedure - message sent by Application Server

- The Application Server sends a message request (i.e. 5GMSGS broadcast message request) to MSGin5G Server. The request includes the information listed in Table 6.7.1.2-1.

Table 6.7.1.2-1: Broadcast message request from Application Server to MSGin5G Server

| Information element | Status | Description |
| --- | --- | --- |
| Originator ID | M | The application level identifier of the message originator. In this message, the originator is the Application Server. |
| Message ID | M | Unique identifier of this message. |
| Service Area ID | C | The identifier of the Service Area where the message needs to be broadcast. This IE is mandatory in the Broadcast Message and is not present in other message scenarios. |
| Payload | M | The payload of the message. |

Editor's Note: If other information of the broadcast messages is found in solutions addressing other KIs, this table need to be updated accordingly.

Editor's Note: The procedure addressing the other requirements, e.g. fragment and store message in MSGin5G Server, will be provided in other solutions. In this solution, the related steps will be omitted

- Upon receiving the message, the MSGin5G Server validates the broadcast message request by checking some conditions, e.g. whether the originator is authenticated and is authorized to send a broadcast message, whether the Service Area ID exists, etc. The MSGin5G Server will go to step 4 if the MSGin5G Server validates the broadcast message request successfully.

- The MSGin5G server sends a broadcast message rejection message to the Application Server. The broadcast message rejection message includes the information listed in Table 6.7.1.2-2. The remaining steps of broadcast message will not be processed.

# Table 6.7.1.2-2: Broadcast message rejection message from MSGin5G Server to Application Server

| Information element | Status | Description |
| --- | --- | --- |
| Originator ID | M | The application level identifier of the message originator. In this message, the originator is the MSGin5G Server. |
| Recipient ID | M | The application level identifier of the message recipient. In this message, the destination is the identifier of the broadcast message originator. |
| Message ID | M | Indicates which message request this message is responding. It is equal to the Message ID included in the broadcast message request which is rejected. |
| Reject Reason | O | This IE contains the failure reason, e.g. the service area does not exist, or the originator is not authorized to send a broadcast message, may be included in this IE. |

- The MSGin5G Server transfers the request to the CBCF and includes at least the Service Area ID and the payload. Furthermore:

- The Start Time is assumed to be immediate.

- The Expiration Time may be configured in the CBCF or in the MSGin5G Server. Alternatively, the Expiration Time is provided as an information element in the broadcast message request.

- The Repetition Interval is assumed to be configured in the CBCF or in the MSGin5G Server. Alternatively, the Repetition Interval is provided as an information element in the broadcast message request.

- The Message Identifier is assumed to be configured in the CBCF or in the MSGin5G Server and is allocated specifically for the MSGin5G service. Alternatively, the Message Identifier is provided as an information element in the broadcast message request or as an information element that can be mapped onto a Message Identifier.

Editor's note: It is FFS if more than one Cell Broadcast Message Identifier is required to differentiate between Application Clients.

- The CBCF broadcasts the message via the 5GS in the service area.

- Since broadcast is downlink-only, any delivery status report needs to be initiated by the MSGin5G Client.

Editor's note: It is FFS if a UE shall initiate sending a delivery status report, since the Application Server may not be aware which UEs were present in the service area to receive the broadcast message.

### 6.7.2	Impacts on existing nodes and functionality

No protocol specification exists in 3GPP for submitting messages to a CBCF.

### 6.7.3	Solution evaluation

The CBCF can be used for transferring the broadcast message if a protocol specification is developed.

# 7	Architectural requirements

## 7.1	Capability requirements

### 7.1.1	General

The application architecture for the MSGin5G Application Service is designed based on the following capabilities:

-	Supported UE types: 5GMSGS UE could be

1.	light weight constrained devices (e.g. sensors, actuators) and

2.	unconstrained devices with advanced capabilities (e.g. washing machine, micro-ovens)

-	Supported communication modes: standalone communication

-	Supported communication payload size: small data message and large data message

-	Supported communication frequency: sporadic and periodic messages

-	Supported communication target: only one target device (point-to-point) and large number of target devices (one-to-many)

-	Supported communication priority type: normal and high priority.

Editor's note: Support for conversation based and session based communication modes are FFS.

Editor's note:	Support for communication priority (e.g. for time sensitivity) including normal and high priority messages is FFS.

### 7.1.2	Requirements

In order to support the capabilities listed above, the application architecture for MSGin5G Application Service shall support the following architecture requirements:

[AR-7.1.2-a]	The application architecture shall enable deployment of 5GMSGS UE which are of heterogeneous nature including light weight constrained devices (e.g. sensors, actuators) and unconstrained devices with advanced capabilities (e.g. washing machine, micro-ovens).

[AR-7.1.2-b]	The application architecture shall enable the standalone communication mode.

[AR-7.1.2-c]	The application architecture shall enable the communication payload size varying from small data message to large data message.

[AR-7.1.2-d]	The application architecture shall enable the communication frequency including sporadic and periodic messages.

[AR-7.1.2-e]	The application architecture shall enable the communication targets varying from only one target device (point-to-point) to a large number of target devices (one-to-many).

[AR-7.1.2-f]	The application architecture shall enable the communication priority including normal and high priority messages.

# 8	Application architecture of the MSGin5G Service

## 8.1	Application architecture principles

The application architecture for MSGin5G Application Service is designed based on the following architecture principles

The

three logical UE types, depending on the type of underlying transport services supported, as follows:

-	5GMSGS UE type - originating/terminating only 5GSMSG transport.

Editor's Note: The 5GMSGS transport protocol is FFS in 3GPP.

NOTE 1: Support for the 5GMSGS transport protocol is not equivalent with support for MSGin5G Application Services. All UE types described in this clause support MSGin5G Application Services

-	Legacy 3GPP UE type - originating/terminating only legacy 3GPP transports, e.g. SMS, NIDD, broadcast, etc.

-	Non-3GPP UE type - originating/terminating only non-3GPP transports.

NOTE 2:  For UEs which do not host a 5GMSGS Application Client, the MSGin5G Server may provide interworking to other messaging services (e.g. SMS) to deliver messages to/from these UEs.

For each of these types, support of the corresponding messaging transports is exclusive, i.e. 5GMSGS UE type supports only 5GMSGS transport and does not support legacy 3GPP transports or non-3GPP transports. Examples of MSGin5G Service deployment scenarios are described in clause 4.

Editor's Note: UEs may be implemented to support a combination of the transport protocols above (e.g. combined  5GMSGS and Legacy 3GPP type). However, specifying functionality for combined types is not in scope.

MSGin5G Application Service supports UE:

-	That could be IMS and non-IMS base

-	a non-IMS UE may be 3GPP UE and non-3GPP UE

-	5GMSGS UE comprises both application client (optional) and 5GMSGS client.

NOTE:	Application service logic in the application client is out of scope of this specification.

## 8.2	Application architecture

Figure 8.2-1 shows the application architecture of the MSGin5G Application Service.  The MSGin5G Application Service shall fulfil the service requirements which are enumerated in 3GPP TS 22.262 [2].

NOTE 1: "5GMSGS" in this document is referred to the new defined transport for the MSGin5G service. The service name in this document uses the "MSGin5G Service" term in 3GPP TS 22.262 [2].

Figure 8.2-1: Application Architecture of the MSGin5G Service

Editor's note:	Representing usage of SEAL in the above application architecture figure in 8.2-1 is FFS.

The 5GMSGS client(s) interacts with SEAL clients over the SEAL-C reference point specified for each SEAL service. The MSGin5G server(s) interacts with SEAL servers over the SEAL-S reference point specified for each SEAL service. The interaction between a SEAL client and the corresponding SEAL server is supported by SEAL-UU reference point specified for each SEAL service as specified in 3GPP TS 23.434 [9].

Editor's note: The MSGin5G Service enhancements to reference points SEAL-C, SEAL-S, SEAL-UU reference points in 3GPP TS 23.434 [9] is FFS.

## 8.3	Functional elements

### 8.3.1	General

The functional entities of the application architecture for the MSGin5G Service are described in this clause.

### 8.3.2	MSGin5G Server

A MSGin5G server provides server-side functionality to assist 5GMSGS clients with the sending and receiving of messages via the MSGin5G Service to/from Application Servers and/or other MSGin5G service endpoints on other UEs.

Functionalities of MSGin5G Server:

-	Interworking with non 3GPP messaging service through the MSGin5G gateway;

-	Exchanging MSGin5G messages with application servers, 5GMSGS clients and MSGin5G gateway; and

-	Providing information related to the MSGin5G Service, such as client availability, to 5GMSGS clients and MSGin5G gateway.

### 8.3.3	5GMSGS Client

A 5GMSGS client provides client-side functionality for UE Application Clients with the sending and receiving of messages via the MSGin5G Service to/from Application Servers and/or other MSGin5G service endpoints; i.e. UEs.

Functionalities of 5GMSGS client:

-	Exposing MSGin5G APIs to enable application clients to use a MSGin5G Service;

-	Suppporting registration of a 5GMSGS client to a MSGin5G Server to enable the exchange of configuration information required to use a MSGin5G Service; and

-	Exchanging MSGin5G messages via a MSGin5G Server to/from Application Servers and/or other MSGin5G service endpoints; i.e. UEs.

### 8.3.4	MSGin5G Gateway

A MSGin5G gateway provides functionality to interwork mesages between the MSGin5G Service and a non-3GPP messaging service.

Functionalities of MSGin5G gateway:

-	Enable a Non-3GPP messaging service to interwork with a MSGin5G Service;

-	Support registration of a MSGin5G Gateway to a MSGin5G Server to enable the exchange of configuration information required to use a MSGin5G Service.

### 8.3.5	Application Client

The application client provides client-side messaging service logic and needed information for the 5GMSGS client to perform MSGin5G service with other endpoints. Using application client to provide this information to the 5GMSGS client is optional as there are other ways to provide this information to the 5GMSGS client such as a native UE implementation with pre-configured information.

### 8.3.6	Application Server

The Application Server supports sending and receiving messages with 5GMSGS service layer protocols and procedures.

### 8.3.7	3GPP Message Client

The 3GPP message client provides client-side functionality for a legacy 3GPP messaging service (e.g. SMS).

NOTE:	The details of the 3GPP message client are out of scope of the present document.

### 8.3.8	Non-3GPP Message Client

The Non-3GPP message client provides client-side functionality for a Non-3GPP messaging service.

NOTE:	The details of the Non-3GPP message client are out of scope of the present document.

### 8.3.9	Non-3GPP Message Server

The Non-3GPP message server provides server-side functionality for Non-3GPP messaging service.

NOTE:	The details of the Non-3GPP message server are out of scope of the present document.

### 8.3.10	SEAL client

The following SEAL clients for MSGin5G Service are supported:

-	Group management client as specified in 3GPP TS 23.434 [9];

-	Configuration management client as specified in 3GPP TS 23.434 [9].

### 8.3.11	SEAL server

The following SEAL servers for MSGin5G Service are supported:

-	Group management server as specified in 3GPP TS 23.434 [9];

-	Configuration management server as specified in 3GPP TS 23.434 [9].

Editor's note: Usage of other SEAL services (e.g. location) from 3GPP TS 23.434 [9] for 5GMSGS Service is FFS.

## 8.4	Reference Points

### 8.4.1	General

The reference points of the service architecture for the MSGin5G Service are described in this clause.

### 8.4.2	MSGin5G-1

The interactions related to enabling MSGin5G message exchange between a 5GMSGS client and a MSGin5G Server are supported by the MSGin5G-1 reference point. This reference point supports:

-	Registration of a 5GMSGS client to a MSGin5G Server when not using IMS base solution; and

-	The exchange of MSGin5G messages.

### 8.4.3	MSGin5G-2

The interactions related to enabling MSGin5G message exchange between a MSGin5G Server and a 3GPP Network are supported by the MSGin5G-2 reference point. This reference point supports:

-	Access to 3GPP Network functions and APIs for retrieval of network capability information; and

-	Access to 3GPP Network functions and APIs to enable sending and receiving of MSGin5G messages.

### 8.4.4	MSGin5G-3

The interactions related to enabling MSGin5G message exchange between an Application Server and a MSGin5G Server are supported by the MSGin5G-3 reference point. This reference point supports:

-	Access to MSGin5G Server and APIs to enable sending and receiving of MSGin5G messages; and

-	Adherence to CAPIF as specified in 3GPP TS 23.222 [13].

### 8.4.5	MSGin5G-4

The interactions related to enabling MSGin5G message exchange between a MSGin5G Gateway Client and a MSGin5G Server are supported by the MSGin5G-4 reference point. This reference point supports:

-	Registration of MSGin5G Gateway to MSGin5G Server; and

-	The exchange of MSGin5G messages.

### 8.4.6	MSGin5G-5

The interactions related to enabling MSGin5G message related information exchange between an application client and a 5GMSGS client are supported by the MSGin5G-5 reference point. This reference point supports:

-	Providing information from application clients required to enable the 5GMSGS client to construct a MSGin5G message to be delivered to other MSGin5G service endpoints.

-	Configuring application clients with information required to enable the 5GMSGS client and MSGin5G Server to exchange and route MSGin5G messages to other MSGin5G service endpoints.

-	Sending notifications and information in the incoming MSGin5G messages received by the 5GMSGS client to the application clients from other MSGin5G service endpoints.

### 8.4.7	MSGin5G-X

The interactions related to enabling MSGin5G message exchange between an application client and a 3GPP message client are supported by this reference point.

Editor's note:	It is FFS whether the MSGin5G-X reference point is in scope of the present document.

### 8.4.8	MSGin5G-Y

The interactions related to enabling MSGin5G message exchange between a 3GPP message client and a 3GPP Network are supported by this reference point.

Editor's note:	It is FFS whether the MSGin5G-Y reference point is in scope of the present document.

### 8.4.9	SEAL-C

The following SEAL-C reference points for MSGin5G Service are supported:

-	GM-C reference point for group management as specified in 3GPP TS 23.434 [9];

-	CM-C reference point for configuration management as specified in 3GPP TS 23.434 [9].

### 8.4.10	SEAL-S

The following SEAL-S reference points for MSGin5G Service are supported:

-	GM-S reference point for group management as specified in 3GPP TS 23.434 [9];

-	CM-S reference point for configuration management as specified in 3GPP TS 23.434 [9].

### 8.4.11	SEAL-UU

The following SEAL-UU reference points for MSGin5G Service are supported:

-	GM-UU reference point for group management as specified in 3GPP TS 23.434 [9];

-	CM-UU reference point for configuration management as specified in 3GPP TS 23.434 [9].

# 9	Overall Evaluation

# 10	Conclusions

Annex A:
5G Architecture considerations

The figures in clause 4 on scenarios show high-level diagrams that need to be mapped onto the 5G architecture as specified in 3GPP TS 23.501 [14] in order to verify if the MSGin5G service can be provided by the 5G architecture. The MSGin5G service architecture as shown in figure A-1 is taken from 3GPP TS 22.262 [2] and shows the consolidated service architecture for all scenarios.

Figure A-1: MSGin5G Service Architecture

UE A is a Controller UE which uses the MSGin5G Service to send messages to Controlled UEs, possibly via the APP Server in case the MSGin5G Server is unable to deal with APP specific issues.

UE B is a Controlled UE that supports the MSGin5G Service.

UE C is a Controlled UE which doesn't support the MSGin5G Service, but supports SMS. The MSGin5G Server converts MSGin5G messages into SMS messages.

UE D is a Controller UE or a Controlled UE which doesn't support the MSGin5G Service, but supports a non-3GPP messaging service.

All UEs (UE A, UE B, UE C and UE D) use the NG-RAN, either via the NR access network or via a non-3GPP access network.

NOTE1: Even though figure A-1 suggest the MSGin5G Gateway is in between the 5G Core or Access Network and UE D, 3GPP TS 22.262 [2] states that UE D is connected via NG-RAN.

NOTE2: Access via E-UTRAN is not in scope of the study.

The APP Server can generate messages in MSGin5G format which are distributed to Controlled UEs by the MSGin5G Server.

Figure A-2 shows the MSGin5G architecture, based on the non-roaming architecture from 3GPP TS 23.501 [14] where the MSGF and MGWF reside in a trusted DN.

Figure A-2: MSGin5G System Architecture for non-roaming

All UEs are connected to the NG-RAN. 5GMSGS UE and Non-3GPP UE are Controller UEs and 5GMSGS UE, Legacy 3GPP UE and Non-3GPP UE are Controlled UEs.

The MSGin5G Server is shown as the MSGin5G Function (MSGF) in figure A-2. The MSGin5G Gateway is shown as the MSGin5G Gateway Function (MGWF) and the APP Server connects directly to the MSGF (in case it supports the MSGin5G Service) or to the MGWF (if it supports a non-3GPP messaging service) and may reside outside the 5GS in an external DN.

The MSGF manages the distribution of the messages it has received from 5GMSGS UE, Non-3GPP UE, from an APP Server, or from the MSGF. The MSGF may use application specific functionality from the APP server that doesn't reside in the MSGF itself.

A CBCF is specified in 3GPP in 3GPP TS 23.041 [6] and can be used for broadcasting messages in the MSGin5G message format to 5GMSGS UE devices and in separate messages in a non-3GPP message format to Non-3GPP UED devices. Even though the CBCF is shown in figure A-2 as a Service Producer network function, it has no service-based interface. This interface could be specified in stage 2 or ATIS-0700008.v002 [12] could be referred to.

The SMSF is specified in 3GPP TS 23.501 [14] and its services can be invoked by the MSGF to send a message as SMS to Legacy 3GPP UE.

Messages towards Non-3GPP UE devices are sent from the MSGF in MSGin5G format to the MGWF and are converted by the MGWF into a non-3GPP message format and also distributed by the MSGF. An APP Server can be used to create messages in the MSGin5G format and the APP Server may receive messages from 5GMSGS UE or Non-3GPP UE. These messages are routed to the MSGF for further distribution to UEs of type 5GMSGS UE, Legacy 3GPP UE or Non-3GPP UE.

An APP Server that doesn't support the MSGin5G Service can send its messages to the MGWF, which converts the message to MSGin5G format before forwarding it to the MSGF.

5GMSGS UE can send its MSGin5G messages to the MSGF via the Control Plane or the User Plane (see Key Issue 5).

Non-3GPP UE can send messages in a non-3GPP message format via the Control Plane or the User Plane (see Key Issue 5) to the MGWF and the MGWF will convert the message into a MSGin5G message format before forwarding it to the MSGF.

Editor's note: Gaps are yet to be identified.

Annex B:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-07 | SA6#32 |  |  |  |  | TR skeleton | 0.0.0 |
| 2019-07 | SA6#32 |  |  |  |  | TR Skeleton agreed in SA6#32: S6-191321 Implemented pCRs approved in SA6#32: S6-191516, S6-191520, S6-191584, S6-191585, S6-191618 Editorial changes by the rapporteur | 0.1.0 |
| 2019-09 | SA6#33 |  |  |  |  | Implemented pCRs approved in SA6#33: S6-191629, S6-191703,S6-191829, S6-191830, S6-191832,S6-191833, S6-191834, S6-191835, S6-191971,S6-191989. Editorial changes by the rapporteur | 0.2.0 |
| 2019-12 | SA6#34 |  |  |  |  | Implemented pCRs approved in SA6#34: S6-192037, S6-192250,S6-192391, S6-192392, S6-192393. Editorial changes by the rapporteur | 0.3.0 |
| 2020-01 | SA6#35 |  |  |  |  | Implemented pCRs approved in SA6#35: S6-200219, S6-200330, S6-200221, S6-200351 Editorial changes by the rapporteur | 0.4.0 |
| 2020-04 | SA6#36 BIS-e |  |  |  |  | Implemented pCRs approved in SA6#36 BIS-e: S6-200456, S6-200614, S6-200585, S6-200461, S6-200521, S6-200612, S6-200615, S6-200610, S6-200611 Editorial changes by the rapporteur | 0.5.0 |
| 2020-06 | SA6#37-e |  |  |  |  | Implemented pCRs approved in SA6#37-e: S6-200825, S6-200923, S6-200942, S6-200943 Editorial changes by the rapporteur | 0.6.0 |
| 2020-07 | SA6#37-e |  |  |  |  | Editorial changes by the rapporteur | 0.6.1 |
| 2020-07 | SA6#37-e |  |  |  |  | Additional formatting corrections by MCC | 0.6.2 |
| 2020-08 | SA6#38-e |  |  |  |  | Implemented pCRs approved in SA6#38-e: S6-201269, S6-201270, S6-201271, S6-201272,S6-201273,S6-201274,S6-201279,S6-201288,S6-201291,S6-201293,S6-20129,S6-201297,S6-201331. Editorial changes by the rapporteur | 0.7.0 |