| 3GPP TR 23.700-27 V0.1.0 (2022-02) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on 5G System with Satellite Backhaul (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Abbreviations	7

4	Architecture Assumptions	7

5	Key Issues	7

5.1	Key Issue #1: PCC/QoS control enhancement considering dynamic satellite backhaul	7

5.1.1	Description	7

5.2	Key Issue #2: Support of Satellite Edge Computing via UPF on board	7

5.2.1	Description	7

5.3	Key Issue #3: Support of Local Data Switching via UPF on-board	8

5.3.1	Description	8

6	Solutions	8

6.0	Mapping of Solutions to Key Issues	8

6.1	Dynamic delay solution	8

6.1.1	Description	8

6.1.2	High level architecture principles	8

6.1.3	Impacts on services, entities and interfaces	10

6.X	Solution for Key Issue #X: <Solution Title>	10

6.X.1	Description	10

6.X.2	Procedures	10

6.X.3	Impacts on services, entities and interfaces	10

7	Overall Evaluation	10

8	Conclusions	10

Annex A: Change history	11

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The scope of this Technical Report is to study the following aspects of 5GS with satellite backhaul:

1:	Architecture enhancements for support of a backhaul with changing delay (e.g. brought by ISL in satellite networks or changed satellite backhaul on the UP path), and possibly limited bandwidth in the case of a gNB with satellite backhaul only (e.g. restricted by the maximum data rate of a satellite beam):

-	Policy/QoS control enhancements based on the detected packet delivery latency and/or bandwidth of the satellite backhaul on the UP path;

-	Exposure of backhaul information to AFs.

2:	Architecture enhancements for support of UPF deployed on GEO satellite with gNB on the ground:

-	Whether and how to enable satellite edge computing services via UPF on-board e.g. to reduce the latency for data transmission, and minimize the backhaul resources consumption;

-	How to enhance local switch for UEs in a communication when they are served by UPF on-board, e.g. to reduce end to end delay comparing with existing 5G LAN local switch at PSA on the ground.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TR 23.737: "Study on architecture aspects for using satellite access in 5G".

# 3	Definitions of terms, symbols and abbreviations

This clause and its three (sub)clauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

# 4	Architecture Assumptions

The 5GS architecture specified in TS 23.501 [2], TS 23.502 [3] and TS 23.503 [4] is used as a baseline.

The following architectural assumptions are applied to the study:

-	At least UP backhaul connection of a gNB is over satellite link.

# 5	Key Issues

## 5.1	Key Issue #1: PCC/QoS control enhancement considering dynamic satellite backhaul

### 5.1.1	Description

Satellite based backhaul is important for remote area or mission critical scenarios, in case it is not possible to build terrestrial backhaul connections.

In Release 17, a satellite backhaul category indication was defined to indicate the satellite backhaul information in. However, this indication may not enough if the satellite backhaul involves multi-hops of ISL or backhaul connections are over different satellites and terrestrial networks. In such case, the capabilities provided by the satellite backhaul can be changed/adjusted dynamically. For example, if inter-satellite links are used on the backhaul path, the delay and bandwidth of the backhaul may be impacted due to the traffic engineering on the links. Satellite backhaul category is not enough to describe such a dynamic satellite backhaul characteristics.

To better serve UEs connecting to 5GC via a gNB with satellite backhaul, the following topics are to be studied:

-	Determination of packet delivery latency or bandwidth or both of the satellite backhaul on the UP path.

-	Policy/QoS control enhancements based on the determined packet delivery latency or bandwidth or both of the satellite backhaul on the UP path.

-	What kind of backhaul information can be exposed to the AF, and how to perform the exposure?

## 5.2	Key Issue #2: Support of Satellite Edge Computing via UPF on board

### 5.2.1	Description

Whether and how to enable satellite edge computing services via UPF on-board e.g. to reduce the latency for data transmission, and minimize the backhaul resources consumption.

NOTE:	Coordination with CT WG4  may be done on aspects related to N4 association between the UPF on board and 5GC NFs on the ground.

## 5.3	Key Issue #3: Support of Local Data Switching via UPF on-board

### 5.3.1	Description

How to support local data switch for UEs in a communication when they are served by UPF on-board, e.g. to reduce end to end delay comparing with existing 5G LAN local switch at PSA on the ground?

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 |  |
| 1 | x |  |  |  |

## 6.1	Dynamic delay solution

### 6.1.1	Description

In Rel-17 reference architecture specified in TS 23.501 [2] and TS 23.502 [3], the AMF is configured with the awareness of satellite backhaul categories as specified in clause 5.8.2.15 of TS 23.501 [2]. The AMF informs the SMF of satellite backhaul category that is being used when PDU session is established and after change or handover causing different backhaul connection to be used. In roaming case, the V-SMF relays the satellite backhaul category to H-SMF. The PCF is informed of satellite backhaul category as the SMF includes it SM Policy Association establishment. Based on this knowledge, the PCF can set the QoS policies accordingly.

NOTE:	For a gNB only has single type of satellite backhaul, the AMF detection of satellite backhaul is based on local configuration, e.g. based on AMF awareness of Global RAN Node ID.

Editor's note:	It is FFS whether and how the detection of satellite backhaul is based on local configuration if a gNB simultaneously has multiple backhaul connections over different satellites on NG-U interface.

Awareness of QoS changes is needed to adapt this Rel-17 solution to the dynamic delay requirement in Rel-18. That awareness of variable delay can be obtained by re-using the capability for the SMF to request QoS monitoring from the UPF that is specified in clause 5.33.3.3 of TS 23.501 [2].

In addition to the already specified use cases where the SMF informs the PCF of the satellite category, the SMF needs to be enhanced to measure the N3 using the echo procedure and to inform the PCF of not only satellite category but also the observed delay.

### 6.1.2	High level architecture principles

Figure 6.1.2-1 shows the call flow that will say the following:

Figure 6.1.2-1: Dynamic delay monitoring and QoS adaptation

1.	During PDU session establishment, PDU session modification or handover that leads to NG-RAN with different backhaul category, the AMF determines the satellite backhaul category based on configuration.

2.	The AMF informs the SMF of satellite backhaul by including the Satellite Backhaul Category in Nsmf_PDUSession_CreateSMContext or in Nsmf_PDUSession_UpdateSMContext message as specified in clause 4.3.2.2 of TS 23.502 [3].

3.	SMF responds by Nsmf_PDUSession_CreateSMContext Responsed message or Nsmf_PDUSession_UpdateSMContext Response.

4.	SMF informs PCF of the satellite backhaul by including Satellite Backhaul Category in Npcf_SMPolicyControl_Update Request during SM Policy Association Establishment as specified in clause 4.16.4 of TS 23.502 [3].

The PCF takes the initial policy decision based on the information that it has received so far but at this point, the PCF is not yet aware of any dynamic changes in the observed backhaul delay.

5.	Since SMF knows based on indication received from the AMF in step 2 that satellite backhaul is used, the SMF initiates GTP-U path monitoring as specified in clause 5.33.3.3 of TS 23.501 [2] no matter whether the GTP-U path is over a satellite backhaul with dynamical delay.

6.	SMF receives GTP-U path monitoring results indicating the observed delay.

7.	Upon receiving the observed delay from the UPF in step 6, the SMF initiates SM Policy Association Modification in order to inform the PCF of the Satellite Backhaul Category and the observed delay.

8.	PCF adjusts the QoS policy according to the observed N3 delay. PCF may need to report the delay to AF for application specific reasons.

### 6.1.3	Impacts on services, entities and interfaces

This solution impacts the following system entities.

SMF:

-	Capability to trigger GTP-U path monitoring when satellite backhaul is used.

-	Capability to report the observed delay resulting from GTP-U path monitoring to PCF

PCF:

-	Capability to receive observed delay and to take it into account for policy control decisions.

## 6.X	Solution for Key Issue #X: <Solution Title>

### 6.X.1	Description

Editor's note:	This clause will describe the solution principles and architecture assumptions for corresponding key issue(s). (Sub-)clause(s) may be added to capture details.

### 6.X.2	Procedures

Editor's note:	This clause describes services and related procedures for the solution.

### 6.X.3	Impacts on services, entities and interfaces

Editor's note:	This clause captures impacts on existing services, entities and interfaces.

# 7	Overall Evaluation

Editor's note:	This clause will provide evaluation of different solutions.

# 8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-02 | SA2#149 | S2-2200380 | - | - | - | Proposed skeleton agreed at SA2#149e | 0.0.0 |
| 2022-02 | SA2#149 | S2-2201593 | - | - | - | Scope of FS_5GSATB. | 0.1.0 |
| 2022-02 | SA2#149 | S2-2201594 | - | - | - | Architectural assumptions of FS_5GSATB. | 0.1.0 |
| 2022-02 | SA2#149 | S2-2201595 | - | - | - | Key Issues on support of satellite edge computing via UPF on board and support of local data switching via UPF on-board. | 0.1.0 |
| 2022-02 | SA2#149 | S2-2201596 | - | - | - | Key issue on PCC/QoS control enhancement considering dynamic satellite backhaul. | 0.1.0 |
| 2022-02 | SA2#149 | S2-2201597 | - | - | - | Dynamic delay solution for key issue#1. | 0.1.0 |