| 3GPP TR 23.700-39 V0.3.0 (2025-09) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on logging and recording of mission critical services, Phase 2; (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	8

3.3	Abbreviations	8

4	Scenarios	8

4.0	General	8

4.1	On-network communications	8

4.1.1	Scenario 1: Recording SIP session-based communications	8

4.1.2	Scenario 2: Recording non-session-based user communications	10

4.1.3	Scenario 3: Recording non-communication related user activities	10

4.1.4	Scenario 4: Recording during interconnection and migration	14

4.2	Off-network communications	14

4.2.1	Scenario 5: Recording off-network communications	14

5	Key Issues	15

5.1	Key Issue 1: Recording of MCVideo one to one pull and push	15

5.2	Key Issue 2: Recording target setting of MCData users and groups	15

5.3	Key Issue 3: Both parties of an MCPTT or MCVideo private call or MCData one-to-one communication are recording targets	16

5.4	Key Issue 4: Recording and replay of e2e encrypted one-to-one communication	16

5.5	Key Issue 5: Recording and replay of e2e encrypted group communication	18

5.6	Key Issue 6: Recording MC Data MSRP sessions	19

5.7	Key Issue 7: Recording HTTP traffic	19

5.8	Key Issue 8: Recording SDS using signalling control plane	19

6	Solutions	20

6.1	Solution 1: Recording of MCVideo one to one pull and push	20

6.1.1	Description	20

6.1.1.1	Functional model and reference points	20

6.1.1.2	Configurations	20

6.1.1.3	Procedures	20

6.1.2	Impacts on existing functional entities and reference points	20

6.1.3	Solution evaluation	20

6.2	Solution 2: Recording target setting of MCData users and groups	20

6.2.1	Description	20

6.2.1.1	Functional model and reference points	20

6.2.1.2	Configurations	21

6.2.1.3	Procedures	21

6.2.2	Impacts on existing functional entities and reference points	21

6.2.3	Solution evaluation	21

6.3	Solution 3 (for KI#6): Recording MC Data MSRP sessions	21

6.3.1	Description	21

6.3.1.0	General	21

6.3.1.1	Functional model and reference points	21

6.3.1.2	Configurations	21

6.3.1.3	Procedures	21

6.3.2	Impacts on existing functional entities and reference points	21

6.3.3	Solution evaluation	22

6.4	Solution 4 (for KI#3): Both parties of an MCPTT or MCVideo private call or MCData one-to-one communication are recording targets	23

6.4.1	Description	23

6.4.1.0	General	23

6.4.1.1	Functional model and reference points	23

6.4.1.2	Configurations	23

6.4.1.3	Procedures	23

6.4.2	Impacts on existing functional entities and reference points	25

6.4.3	Solution evaluation	25

7	Overall evaluation	25

8	Conclusions	25

Annex A (informative): List of stage 1 requirements	26

A.1	Requirements for on-network recording and audit	26

A.2	Requirements for off-network recording and audit	26

Annex B (informative): Mapping of stage 1 requirements to solutions	28

Annex C (informative): Change history	29

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document studies solutions to satisfy the requirements for logging and recording of mission critical services. It identifies enhancements to be included in the technical specifications for MC Common functional architecture (3GPP TS 23.280 [2]), MCPTT (3GPP TS 23.379 [3]), MCVideo (3GPP TS 23.281 [4]), MCData (3GPP TS 23.282 [5]), MC Interworking with LMR (3GPP TS 23.283 [6]), and Mission Critical services over 5G System (3GPP TS 23.289 [7]). The requirements for this study are taken from the Stage 1 requirements in 3GPP TS 22.280 [8].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.280: "Common functional architecture to support mission critical services; Stage 2".

[3]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[4]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[5]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[6]	3GPP TS 23.283: " Mission Critical Communication Interworking with Land Mobile Radio Systems; Stage 2".

[7]	3GPP TS 23.289: "Mission Critical services over 5G System; Stage 2".

[8]	3GPP TS 22.280: "Mission Critical Services Common Requirements (MCCoRe); Stage 1".

[9]	3GPP TS 33.180: "Security of the Mission Critical (MC) service".

# 3	Definitions of terms, symbols and abbreviations

This clause and its three (sub) clauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1], 3GPP TS 23.280 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1] or 3GPP TS 23.280 [2].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

# 4	Scenarios

## 4.0	General

The scenarios in the following clauses cover all ‘categories’ of logging/recording required in the SA1 user requirements. Each scenario lists all related procedures (clauses) in MCPTT/MCVideo/MCData TSs to guide the development of Key Issues and Solutions.

“Category 1” i.e. Scenario 1 – Recording SIP session-based communications – was partially solved in Rel-19. The main topic for KIs and solutions in this category should be to check/re-check that all the listed features/procedures can be recorded using the SIP session recording (i.e. SIPREC). However, some new issues are also expected in this ‘category’, e.g. simultaneous sessions.

For all the other categories it is anticipated that additional functionality and/or changes to the functional architecture will be required into the MC system to enable recording. These additions shall be studied and solved with related KIs and solutions in this study.

## 4.1	On-network communications

### 4.1.1	Scenario 1: Recording SIP session-based communications

This scenario lists the features/procedures/clause numbers that are relevant for recording SIP session-based user communications. These clauses may either have impact to the recording solution(s) and/or may need to be updated due to the recording feature.

- MC Common, 3GPP TS 23.280 [2]

  - 10.3	Pre-established session (on-network)

  - 10.4	Simultaneous session (on-network)

    - The related KI needs to study e.g. whether simultaneous sessions can be recorded with one recording (SIPREC) session or if each call/session requires a dedicated recording session.

- MCPTT, 3GPP TS 23.379 [3]

  - 10.5	Pre-established session (on-network)

  - 10.6	Group call

    - 10.6.2	On-network group call

      - Basic recording functionality for on-network group calls is in Stage2 Rel-19

      - The related KIs must cover all those procedures from this clause that may need some additional functionality and/or parameters for recording.

  - 10.7	Private call

    - 10.7.2	Private call in on-network

      - Basic recording functionality for on-network private calls is in Stage2 Rel-19

      - The related KIs must cover all those procedures from this clause that may need some additional functionality and/or parameters for recording.

    - The supplementary services in 10.7.4, 10.7.5 and 10.7.6 in TS 23.379 [3] are also procedures that need to be recorded

  - 10.8	Simultaneous session for MCPTT calls (on-network)

    - See 10.4 of 3GPP TS 23.280 [2] above.

  - 10.9	Floor control

  - 10.14	Ambient listening call

  - 10.16	Remotely initiated MCPTT call

  - 10.19 Ad hoc group call

- MCVideo, 3GPP TS 23.281 [4]

  - 7.1.2 	On-network group call

    - Basic recording functionality in Stage2 Rel-19

  - 7.2.2	Private call on-network

    - Basic recording functionality in Stage2 Rel-19

  - 7.3.2	On-network video pull

  - 7.3.2.3	One-to-one video pull

    - This is a private video call between users.

  - 7.3.2.4	One-from-server video pull

    - This is a private video call between server and client.

  - 7.4.2	On-network video push

  - 7.4.2.3	One-to-one video push

    - This is a private video call between users.

  - 7.4.2.4	One-to-server video push

    - This is a private video call between server and client.

  - 7.4.2.5	Remotely initiated video push

  - 7.4.2.6	Remotely initiated video push to group

  - 7.6		Ambient viewing call

  - 7.7.1	Transmission control for on-network MCVideo service

  - 7.7.1.3	Transmission control within one MC system for MCVideo service

  - 7.11	Simultaneous session for MCVideo calls (on-network)

    - See 10.4 of 3GPP TS 23.280 [2] above.

  - 7.17	MCVideo adaptation during MCVideo communication

  - 7.19	Ad hoc group call

    - 7.19.3.1	Ad hoc group call procedures in single MCVideo system

- MCData, 3GPP TS 23.282 [5]

  - 7.4.2	Short data service for on-network

    - 7.4.2.3	One-to-one standalone short data service using media plane

    - 7.4.2.4	One-to-one short data service session

    - 7.4.2.6	Group standalone short data service using media plane

    - 7.4.2.7	Group short data service session

    - The emergency and imminent peril upgrade/cancel procedures 7.4.2.8 – 7.4.2.12 shall be recorded as part of the ongoing SDS session.

  - 7.5.2	File distribution for on-network

    - 7.5.2.4	One-to-one file distribution using HTTP

      - Sending HTTP link to an uploaded file in a SIP session

    - 7.5.2.5	One-to-one file distribution using media plane

    - 7.5.2.7	Group standalone file distribution using media plane

    - The emergency and imminent peril upgrade/cancel procedures 7.5.2.11 – 7.4.2.15 shall be recorded as part of the ongoing FD session.

  - 7.6	 Transmission and reception control

  - 7.7 Communication release

  - 7.8	 Conversation management

    - 7.8.2	Conversation management for on-network

  - 7.14	IP Connectivity

  - 7.17	Ad hoc group data communication

### 4.1.2	Scenario 2: Recording non-session-based user communications

This scenario lists the features/procedures/clause numbers that are relevant for recording non-session-based user communications. These clauses may either have impact to the recording solution(s) and/or may need to be updated due to the recording feature.

- MC Common, 3GPP TS 23.280 [2]

  - (nothing)

- MCPTT, 3GPP TS 23.379 [3]

  - (nothing)

- MCVideo, 3GPP TS 23.281 [4]

  - (nothing)

- MCData, 3GPP TS 23.282 [5]

  - 7.4.2	Short data service for on-network

    - 7.4.2.2	One-to-one standalone short data service using signalling control plane

    - 7.4.2.5	Group standalone short data service using signalling control plane

  - 7.5.2	File distribution for on-network

    - 7.5.2.2	File upload using HTTP

    - 7.5.2.3	File download using HTTP

    - 7.5.2.4	One-to-one file distribution using HTTP

    - 7.5.2.6	Group standalone file distribution using HTTP

    - 7.5.2.10	Group standalone file distribution using the MBMS download delivery method

  - 7.9.3	Enhanced status for on-network

    - Same procedure as 7.4.2.5 Group standalone short data service using signalling control plane

### 4.1.3	Scenario 3: Recording non-communication related user activities

This scenario lists the features/procedures/clause numbers that are relevant for recording non-communication related user activities. These clauses may either have impact to the recording solution(s) and/or may need to be updated due to the recording feature.

- MC Common, 3GPP TS 23.280 [2]

  - 10.1	MC service configuration

    - Not all MC service configuration procedures/functions are listed in the related user requirements [R-6.15.4-006], [R-6.15.4-007] and [R-6.15.4-009] in 3GPP TS 22.280 [8].

    - However, several procedures from this clause are listed, e.g. “user remote logout”, “failed authorization attempts”, “regroup operations”, “affiliation change” etc have to be recorded.

    - To keep things simple, it is better to conclude that all “MC service configuration” procedures are subject to recording (for a user who is a recording target).

    - This will have architectural impacts i.e. a recording client is needed also in the CMS and GMS. These need to be studied in the related KI(s).

    - See also Scenario 4 for interconnection/migration

  - 10.2	Group management (on-network)

    - Same as 10.1 above, only some of the group management procedures are listed in the related user requirements [R-6.15.4-006] and [R-6.15.4-007] in 3GPP TS 22.280 [8], but to avoid any extra complexity, it is better to conclude that all group management procedures are subject to recording.

    - The related KI(s) need to cover at least the following issues:

    - 10.2.3	Group creation

      - If a target user is in the “MC service ID list” of the Group creation request (10.2.2.1) this group shall be marked as target for recording

    - 10.2.4	Group regrouping

      - 10.2.4.1	Temporary group formation - group regrouping within an MC system

      - 10.2.4.2a	Temporary group tear down within an MC system

        - If a target group is part of group regrouping/tear down operations, the signalling/metadata shall be recorded.

        - Architectural issue with two options – in the procedures where GMS sends a notification to MC service server, which entity records the procedure – GMS or MC service server (or both).

      - See also Scenario 4 for interconnection/migration.

    - 10.2.5	Membership and affiliation list query

      - Recording this procedure is obvious when a recording target (user) sends the query. However, what if the request comes from a user that is not a recording target, but the group (or a member of the group) which information is requested is a recording target? This needs to be studied/concluded in the related KI(s).

    - 10.2.6	Group membership

      - Same issue as in 10.2.4.2a above. If/when a recording client exists both in the GMS and MC service server, which entity records the event?

      - On group membership change, the GMS needs to evaluate whether the change has impact on the group as a target or not-as-a target for recording.

    - 10.2.8	Group deletion

      - Same issue as in 10.2.4.2a above. If/when a recording client exists both in the GMS and MC service server, which entity records the event?

      - Group deletion will also trigger removal of affiliations. As per SA1 user requirements, this affiliation change – if the user is a target for recording - shall be recorded.

    - See also Scenario 4 for interconnection/migration.

  - 10.5	Use of UE-to-network relay

    - Since the recording is done in the server(s), the recording process is not impacted on the possible usage of UE-to-network relay(s) between the target user and the server(s).

  - 10.6	General user authentication and authorization for MC services

    - 10.6.1	Primary MC system

      - As per SA1 user requirements, e.g. failed authorization attempts, and registration state change are subject to recording. This will have architectural impacts that need to be studied in the related KI(s) (a recording client is needed in the IdMS).

    - See also Scenario 4 for interconnection/migration.

  - 10.7	Use of MBMS transmission

    - There is no explicit SA1 requirements for recording MBMS related signalling. However, requirement [R-6.15.4-010] in 3GPP TS 22.280 [8] on the “network access events” should cover this -> all MBMS related signalling shall be recorded (for the target users).

      - These procedures are one example where signalling may happen both during an active user communication session – i.e. when a recording session is active - and also when the (target) user has no communication session active. This may (or not) have impact on the solutions of this study.

    - The actual use of MBMS vs unicast should have no impact to the recording of user activities and transmissions.

  - 10.8	Affiliation and de-affiliation to/from MC service group(s)

  - 10.9	Location management (on-network)

    - The related KI(s) needs to study the architectural impacts of recording location signalling, i.e. a recording client in the LMS is needed.

NOTE: CT1 updates to MC location protocol (Rel-19) must be taken into account in the related KIs/solutions.

    - See also Scenario 4 for interconnection/migration.

  - 10.10	Emergency Alert

    - 10.10.1	On-network emergency alert

    - 10.10.3	Ad hoc group emergency alert

  - 10.11	MC service resource management (on-network)

  - 10.12	Priority Management

    - The priority mgmt procedures between MC service servers should have no impact on the recording of user activities. I.e. these procedures shall not be recorded.

  - 10.13	Functional alias management (on-network)

  - 10.15	Preconfigured regrouping

    - See also Scenario 4 for interconnection/migration.

  - 10.17	Sharing administrative configuration between interconnected MC systems

    - Not in the scope of recording.

  - 11	MC gateway UE

    - MCGWUE should have no impact to recording. However, if needed, a KI can be added to study this topic.

- MCPTT, 3GPP TS 23.379 [3]

  - 10.1	MCPTT service configuration (common)

    - See 10.1 in 3GPP TS 23.280 [2].

  - 10.2	User authentication and authorization for MCPTT service (common)

    - See 10.6 in 3GPP TS 23.280 [2].

  - 10.3	Affiliation and de-affiliation to/from MCPTT group(s) (common)

    - Shall be recorded. See 10.8 in 3GPP TS 23.280 [2].

  - 10.3a	Activation and de-activation of/from functional alias(es)

    - Shall be recorded. See 10.13.4 and 10.13.5 in 3GPP TS 23.280 [2].

  - 10.4	MCPTT group selection

    - Shall be recorded

  - 10.5	Pre-established session (on-network)

    - Shall be recorded, see also 10.3 in 3GPP TS 23.280 [2].

  - 10.17	Support for multiple devices

    - KI is needed to study possible impact of multiple devices to recording.

- MCVideo, 3GPP TS 23.281 [4]

  - 7.5.2	On-network capability information sharing

    - FFS if this needs to be recorded?

  - 7.8		MCVideo service configuration

    - See 10.1 of 3GPP TS 23.280 [2] above

  - 7.9		Affiliation and de-affiliation to/from MCVideo group(s)

    - See 10.8 of 3GPP TS 23.280 [2] above

  - 7.9a	Functional alias management

    - See 10.13 of 3GPP TS 23.280 [2] above

  - 7.10	Use of MBMS transmission (on-network)

    - See 10.7 of 3GPP TS 23.280 [2] above

  - 7.12	User authentication and authorization for MCVideo service

    - See 10.6 of 3GPP TS 23.280 [2] above

  - 7.13	Support for multiple devices

    - KI is needed to study possible impact of multiple devices to recording. Can be the same KI as for MCPTT.

  - 7.14	Location information (on-network)

    - See 10.9 of 3GPP TS 23.280 [2] above

  - 7.15	MCVideo resource management (on-network)

    - See 10.11 of 3GPP TS 23.280 [2] above

  - 7.16	MCVideo client query

    - FFS if this procedure needs to be recorded.

- MCData, 3GPP TS 23.282 [5]

  - 7.1		MCData service configuration

    - See 10.1 in 3GPP TS 23.280 [2] above

  - 7.2		Affiliation and de-affiliation to/from MCData group(s)

    - See 10.8 of 3GPP TS 23.280 [2] above

  - 7.3		Use of MBMS transmission (on-network)

    - See 10.7 of 3GPP TS 23.280 [2] above

  - 7.10	MCData emergency alert (on-network and off-network)

    - See 10.10 of 3GPP TS 23.280 [2] above

  - 7.10a	MCData ad hoc group emergency alert (on-network)

    - See 10.10 of 3GPP TS 23.280 [2] above

  - 7.11	User authentication and authorization for MCData service

    - See 10.6 of 3GPP TS 23.280 [2] above

  - 7.12	MCData resource management (on-network)

    - See 10.11 of 3GPP TS 23.280 [2] above

  - 7.13	Operations on MCData message store

    - The related KI needs to study architectural impacts of recording message store operations i.e. whether a recording client is needed in the message store.

  - 7.15	Location information (on-network)

    - See 10.9 of 3GPP TS 23.280 [2] above

- MC Security 3GPP TS 33.180 [9]

  - Key management

### 4.1.4	Scenario 4: Recording during interconnection and migration

This scenario lists the features/procedures/clause numbers that are relevant for recording during interconnection and migration. These clauses may either have impact to the recording solution(s) and/or may need to be updated due to the recording feature.

- MC Common, 3GPP TS 23.280 [2]

  - 5.2.9		Migration

  - 5.2.10		Interconnection

  - 7.3.1.2	Functional model for an MC system

      - The related KI(s) need to study if Recording servers between two MC systems need to have a interface/reference point between them.

  - 7.19.3.2	Ad hoc group call involving multiple MC systems (SIP session)

  - 10.1.1.2	MC service configuration for migration to partner MC system

  - 10.1.4.3.2 MC service user receiving MC service from a partner MC system

  - 10.2.4.2	Temporary group formation involving multiple MC systems

  - 10.2.4.3	Temporary group tear down involving multiple group host servers

  - 10.2.7		Group configuration for interconnection

  - 10.6.2		Interconnection partner MC system

  - 10.6.3		Migration to partner MC system

  - 10.9.3.10 	Usage of location information across MC systems procedure

  - 10.14		Generic procedures for interconnection

  - 10.15.3.3 	Preconfigured regroup procedures in multiple MC systems

  - 10.16		Generic procedures for migration

- MCPTT, 3GPP TS 23.379 [3]

  - A.3	MCPTT user profile configuration data

    - The related KI(s) need to study potential impacts to user configuration data vs recording when migrated.

- MCVideo, 3GPP TS 23.281 [4]

  - A.3	MCVideo user profile configuration data

    - The related KI(s) need to study potential impacts to user configuration data vs recording when migrated.

- MCData, 3GPP TS 23.282 [5]

  - A.3	MCData user profile configuration data

    - The related KI(s) need to study potential impacts to user configuration data vs recording when migrated.

## 4.2	Off-network communications

### 4.2.1	Scenario 5: Recording off-network communications

This scenario lists the features/procedures/clause numbers that are relevant for recording during off-network. These clauses may either have impact to the recording solution(s) and/or may need to be updated due to the recording feature.

- MC Common, 3GPP TS 23.280 [2]

  - 7.3.2	Off-network functional model

  - 10.10.2	Off-network emergency alert

  - A.4	Group configuration data

- MCPTT, 3GPP TS 23.379 [3]

  - 7.3.2	Off-network functional model

  - 10.6.3	Off-network group call

  - 10.6.3.9.3 MCPTT emergency alert (off-network)

    - See 10.10.2 of 3GPP TS23.280 [2] above

  - 10.7.3	Private call in off-network

  - 10.9.2	Floor control for off-network MCPTT service

  - A.3	MCPTT user profile configuration data

- MCVideo, 3GPP TS 23.281 [4]

  - 6.1.2	Off-network functional model

  - 7.1.3.9	MCVideo emergency alert (client)

    - See 10.10.2 of 3GPP TS23.280 [2] above

  - 7.1.3	Off-network group communications

  - 7.2.3	Off-network private communications

  - 7.3.3	Off-network video pull

  - 7.4.3	Off-network video push

  - 7.5.3	Off-network capability information sharing

  - 7.7.2	Off-network transmission control

  - 7.18	Use of ProSe capabilities in off-network MCVideo communications

  - A.3	MCVideo user profile configuration data

- MCData, 3GPP TS 23.282 [5]

  - 7.4.3	Short data service for off-network

  - 7.5.3	File distribution for off-network

  - 7.8.3	Conversation management for off-network

  - 7.9.4	Enhanced status for off-network

  - 7.10	MCData emergency alert (on-network and off-network)

    - See 10.10.2 of 3GPP TS23.280 [2] above

  - 7.16	Use of ProSe capabilities in off-network MCData communications

  - A.3	MCData user profile configuration data

# 5	Key Issues

## 5.1	Key Issue 1: Recording of MCVideo one to one pull and push

MC Video one to one pull and push procedures are MC Video private call with specific parameter values. As described in the scenario (4.1.1), the required procedures, information flows and parameters already exist in the Rel-19 MCX specifications.

To fully support this scenario, the following aspects need to be studied:

1.	Consider adding into 3GPP TS 23.280 [2] an explicit list of services that can be recorded with the existing procedures and reference points i.e. REC-4 and SIPREC. Remove all Rel-19 specific notes, see e.g. 3GPP TS 23.280 [2], clause 7.5.2.34.

## 5.2	Key Issue 2: Recording target setting of MCData users and groups

Functionality required for recording of MCData services was not completed in Rel-19. Those parts that were ready, including setting MCData users/groups as targets, were removed from the Rel-19 TSs, to avoid any misunderstandings of the release content.

Now this functionality needs to be added back.

Issues:

-	Revert the changes (i.e. removals) done for the Rel-19 TSs

-	Study if some additional parameters for MCData recording targets are required

## 5.3	Key Issue 3: Both parties of an MCPTT or MCVideo private call or MCData one-to-one communication are recording targets

This key issue focuses on the case when, in a MCPTT or MCVideo private call, MCData one-to-one communication or MCData IP connectivity, both parties are targets for recording.

The issues / alternatives to be studied are:

-	Starting only one recording session

-	Starting a separate recording session for each MCPTT or MCVideo private call, MCData one-to-one communication or point-to-point participant

## 5.4	Key Issue 4: Recording and replay of e2e encrypted one-to-one communication

Figure 5.4-1: Functional elements relevant for recording and replay of e2e encrypted one-to-one communication

This Key issue focuses on the case when a private call or one-to-one communication between MC client A and MC client B is e2e encrypted.

A basic assumption is that the Private Communication Key (PCK) is known only to the communication parties (client A and B) and, at some point of time, also to the Recording server. To ensure e2e confidentiality, as required in [R-6.15.4-005] of TS 22.280 [2], the PCK shall never be revealed to any external entity, including the Replay clients.

There are two options to record e2e encrypted media:

1)	Decrypt the media in the Recording server before storage. This option requires that the media, before storage, is also re-encrypted with some implementation specific mechanisms.

2)	Store the recorded media as e2e encrypted. This option requires that, when a replay is requested by an authorized user at some later point of time, the Recording server shall be able to fetch/generate the used PCK to decrypt the media.

In both alternatives, for the replay of the recordings, a secure connection between the Recording server and the Replay client is required (REC-1), but this connection and the security mechanisms used are outside the scope of 3GPP.

Issues:

-	Study clause 5.6 of 3GPP TS 33.180 [9]. Is there something that should be referred to in 3GPP TS 23.280 [2], 3GPP TS 23.281 [4], 3GPP TS 23.282 [5] or 3GPP TS 23.379 [3]?

-	How does the Recording server get the PCK? Study both options 1 and 2 (see above) and define – if needed - the required procedures, information flows and parameters.

NOTE:	This KI will require co-operation with SA3 and the solutions may require changes also to TS 33.180 [9].

## 5.5	Key Issue 5: Recording and replay of e2e encrypted group communication

Figure 5.5-1: Functional elements relevant for recording and replay of e2e encrypted group communication

This Key issue focuses on the case when group communication is e2e encrypted.

A basic assumption is that the Group Master Key (GMK) is known only to the group members and, at some point of time, also to the Recording server. To ensure e2e confidentiality, as required in [R-6.15.4-005] of TS 22.280 [2], the GMK shall never be revealed to any external entity, including the Replay clients.

There are two alternatives to record e2e encrypted group media:

1)	Decrypt the media in the Recording server before storage. This option requires that the media, before storage, is re-encrypted with some implementation specific mechanisms.

2)	Store the recorded media as e2e encrypted. This option requires that, when a replay is requested by an auditor at some later point of time, the Recording server shall be able to fetch/generate the used GMK and decrypt the media.

In both alternatives, for the replay of recordings, a secure connection between the Recording server and the Replay client is required (REC-1), but this connection and the security mechanisms used there are outside the scope of 3GPP.

Issues:

-	Study clause 5.7 of 3GPP TS 33.180 [9]. Is there something that should be referred to in 3GPP TS 23.280 [2], 3GPP TS 23.281 [4], 3GPP TS 23.282 [5] or 3GPP TS 23.379 [3]?

-	How does the Recording server get the GMK? Study both options 1 and 2 (see above) and define – if needed - the required procedures, information flows and parameters

NOTE:	This KI will require co-operation with SA3 and the solutions may require changes also to 3GPP TS 33.180 [9].

## 5.6	Key Issue 6: Recording MC Data MSRP sessions

MC Data SDS and FD can use either SIP MESSAGEs (in signalling control plane) or MSRP on a SIP session (media plane).

It needs to be studied how to record MSRP sessions. This study shall also consider impacts to MC functional architecture and reference points, including clauses in 3GPP TS 23.280 [2] and 3GPP TS 23.282 [5] that need to be updated.

## 5.7	Key Issue 7: Recording HTTP traffic

Scenario 2 lists (MC Data) non-SIP-session-based features/procedures, including file distribution procedures using HTTP.

Scenario 3 lists all non-communication related procedures, many of those utilizing HTTP signalling.

It needs to be studied how HTTP traffic in MC system can be recorded. This study shall also consider impacts, if any, to MC functional architecture and reference points, including clauses in 3GPP TS 23.280/281/282/379 [2][4][5][3] that need to be updated. Possible updates to 3GPP TS 33.180 [9] shall be coordinated with SA3.

## 5.8	Key Issue 8: Recording SDS using signalling control plane

Recording of SDS using signalling control plane for private and group communications needs to be studied (see scenario 2).

This study shall also consider impacts to MC functional architecture and reference points, including clauses in 3GPP TS 23.280 [2] and/or 3GPP TS 23.282 [5] that need to be updated.5.9	Key Issue 9: Recording of non-communication related user activities for MC services

The recording of non-communication related user activities refers to the scenario 3.

It needs to be studied whether existing reference point i.e. REC-4 defined in 3GPP TS 23.280 [2], clause 7.5.2.34 can be extended to support recording of non-communication related MC service procedures.

It also needs to be studied whether new procedures need to be defined in 3GPP TS 23.280 [2], 3GPP TS 23.379 [3], 3GPP TS 23.281 [4], 3GPP TS 23.282 [5] to support recording of non-communication related MC service procedures.

# 6	Solutions

## 6.1	Solution 1: Recording of MCVideo one to one pull and push

### 6.1.1	Description

#### 6.1.1.1	Functional model and reference points

As described in Key issue 1 and Scenario 4.1.1, MC Video one to one pull and push procedures are MC Video private call with specific parameter values. I.e. the existing functionality in the Rel-19 TSs can be utilized for this service.

#### 6.1.1.2	Configurations

None.

#### 6.1.1.3	Procedures

None.

### 6.1.2	Impacts on existing functional entities and reference points

7.5.2.34	Reference point REC-4 (between recording server and MC service servers)

The REC-4 reference point, which exists between recording server and MC service servers, is used for transmitting metadata and media of the communication sessions of target users and target groups.

NOTE:	In this version of this specification the communication sessions that may be recorded are MCPTT and MCVideo group and private calls.

The REC-4 reference point shall use the SIP-2 reference point for transport and routing of signalling and communication sessions related metadata. If an MC service server and a recording server are served by different SIP cores, then the REC-4 reference point shall also use the SIP-3 reference point for transport and routing of signalling and communication sessions related metadata.

MC service server and recording server shall use Session Recording Protocol as specified in IETF RFC-7866 [40] for establishing and controlling recording sessions between MC service servers and recording server for MCPTT and MCVideo private and group communications. IETF RFC-7866 [40] specifies the use of SIP, Session Description Protocol (SDP), and RTP to establish a recording session.

The required changes/amendments for the Rel-19 TSs are clarifications. This solution proposes the following changes to TS 23.280 clause 7.5.2.34.

### 6.1.3	Solution evaluation

TBD.

## 6.2	Solution 2: Recording target setting of MCData users and groups

### 6.2.1	Description

#### 6.2.1.1	Functional model and reference points

This solution has no impact on the existing functional model or reference points.

#### 6.2.1.2	Configurations

The removal of MCData recording target configurations from Rel-19 - S6-250334 (TS 23.282) and S6-250335 (TS 23.280) - shall be reverted.

#### 6.2.1.3	Procedures

This solution has no impact on the existing procedures.

### 6.2.2	Impacts on existing functional entities and reference points

This solution has no impact on the existing functional model or reference points.

### 6.2.3	Solution evaluation

TBD.

## 6.3	Solution 3 (for KI#6): Recording MC Data MSRP sessions

### 6.3.1	Description

#### 6.3.1.0	General

MC Data SDS and FD can use either SIP MESSAGEs (in signalling control plane) or MSRP on a SIP session (media plane).

#### 6.3.1.1	Functional model and reference points

See 6.3.2.

#### 6.3.1.2	Configurations

No changes.

#### 6.3.1.3	Procedures

No changes.

### 6.3.2	Impacts on existing functional entities and reference points

For the existing reference points, Figure 7.3.1.2-3 in 3GPP TS 23.280 [2] (Relationships between reference points of MC service application plane and signalling control planes) needs a small update i.e. adding MSRP to the REC-4 reference point, see Figure 6.3.2-1.

Figure 6.3.2-1 is an updated Figure 7.3.1.2-3 in 3GPP TS 23.280 [2]. The updates are in blue font.

Figure 6.3.2-1. Relationships between reference points of MC service application plane and signalling control planes

Editor's note: Other impacts on 3GPP TS 23.280 [2] and impacts on 3GPP TS 23.282 [5], if any, are FFS.

### 6.3.3	Solution evaluation

FFS.

## 6.4	Solution 4 (for KI#3): Both parties of an MCPTT or MCVideo private call or MCData one-to-one communication are recording targets

### 6.4.1	Description

#### 6.4.1.0	General

As described in 3GPP TS 23.280 [2] clause 10.18.3.2 (Procedure for recording communication sessions of an MC service user), a (SIPREC) recording session is started when a communication session involving recording target user is initiated.

If both parties of a private cal / one-to-one session are recording targets, there are two options – record them using the same (SIPREC) recording session or start two separate recording sessions.

Both options have their pros and cons. However, for this issue 3GPP specifications should not require a specific solution i.e. this detail shall be left for implementation.

However, to guide CT1 and to avoid this issue popping up again in few years, we should add a NOTE indicating that the issue has been concluded.

#### 6.4.1.1	Functional model and reference points

No changes.

#### 6.4.1.2	Configurations

No changes.

#### 6.4.1.3	Procedures

A new NOTE is proposed to 3GPP TS 23.280 [2], see below (new text bold&underlined):

TS 23.280 / 10.18.3.2	Procedure for recording communication sessions of an MC service user

Figure 10.18.3.2-1 below illustrates the recording procedure for communication sessions of an MC service user (MCPTT ID, MCVideo ID).

Pre-conditions:

-	The user (MCPTT ID or MCVideo ID) has been configured as a target for recording in the MC service user profile (see 3GPP TS 23.281 [12] clause A.3 and 3GPP TS 23.379 [16] clause A.3) and the user profile has been uploaded to the MC service server.

-	The target MC service user has performed MC user authentication and service authorization (see 3GPP TS 33.180 [25], clause 5.1.1 and clause 5.1.2) and has fetched its user profile(s) from the configuration management server.

Figure 10.18.3.2-1: Recording procedure for a target MC service user

1.	An MC communication session (MCPTT/MCVideo private call) involving the target user is initiated.

2.	A Session Recording Client (SRC) in the MC service server starts a recording session (see IETF RFC 7866 [40]) with the recording server.

NOTE 1:	If both parties are recording targets, the SRC may start either one recording session or two recording sessions (one for each party). This decision is out of scope of 3GPP.

3.	The MC communication session ends.

4.	The recording session ends.

5.	The recording server stores the recording session, and it is available for download and/or replay.

NOTE 2:	Steps 1-5 are repeated as long as none of the conditions of step 6 is true.

6.	When the user (MCPTT ID / MCVideo ID) is removed from the recording targets or the user logs off from the MC service or the UE is switched off, an ongoing recording session (if exists) is terminated.

### 6.4.2	Impacts on existing functional entities and reference points

None.

### 6.4.3	Solution evaluation

FFS.

# 7	Overall evaluation

# 8	Conclusions

Annex A (informative):
List of stage 1 requirements

# A.1	Requirements for on-network recording and audit

Requirements for on-network recording and audit in 3GPP TS 22.280 [8] are listed in Table A.1-1. They are applied to all services MCPTT, MCVideo and MCData according to the reference tables in Annexes A, B and C of 3GPP TS 22.280 [8].

Table A.1-1: Requirements for on-network recording and audit

| Requirement | Description |
| --- | --- |
| [R-6.15.4-001] | The MCX Service shall provide a mechanism for a Mission Critical Organization to log the metadata of the MCX Service Group Communications and MCX Service Private Communications under the organization's authority. |
| [R-6.15.4-002] | Metadata shall be logged for both the transmitting Participant and the receiving Participant(s). |
| [R-6.15.4-003] | The MCX Service shall provide a mechanism for a Mission Critical Organization to record the transmissions of the Group Communications and Private Communications under the organization's authority. |
| [R-6.15.4-004] | The MCX Service shall provide a mechanism for a Mission Critical Organization to log at least the following metadata per communication: depending on service this may include; start time, date, MCX User ID, functional alias(es), MCX Group ID, Location information of the transmitting Participant, end time or duration, end reason, type of communication (e.g., MCX Service Emergency, regroup, private) and success/failure indication. |
| [R-6.15.4-005] | If an MCX Service Group Communication or MCX Service Private Communication uses end-to-end confidentiality, the MCX Service shall provide a mechanism for a Mission Critical Organization to maintain the end-to-end confidentiality when the MCX Service Group Communication or MCX Service Private Communication is logged. |
| [R-6.15.4-006] | The MCX Service shall provide a mechanism for a Mission Critical Organization to log the metadata of non-communication related user activities under the agency's authority. |
| [R-6.15.4-007] | The MCX Service shall provide a mechanism for a Mission Critical Organization to log at least the following non-communication activity types: MCX Service Emergency Alert, MCX Service Emergency Alert cancellation, In-progress Emergency cancellation, registration state change, overridden event, user remote logout, changing another user's affiliations, affiliation change, change of Selected MCX Service Group, (de)activation of functional alias(es) and User and Group Regroup operations. |
| [R-6.15.4-008] | The MCX Service shall provide a mechanism for a Mission Critical Organization to log at least the following metadata per non-communication activity: time, date, MCX Service User identity, activity type and pertinent information about the activity (e.g. MCX User/Group ID(s) included in the temporary group created by the User/Group Regroup operation). The following metadata should be logged if applicable to the activity type: MCX Service Group ID, Location information of the MCX User, affiliation list, target MCX Service User ID and success/failure indication. |
| [R-6.15.4-009] | The MCX Service shall provide a mechanism for a Mission Critical Organization to log metadata for all failed authorization attempts (e.g., invalid login password) by an MCX User. |
| [R-6.15.4-010] | The MCX Service shall provide a mechanism to collect metadata for network access events (e.g., pre-emption of the 3GPP system bearer services, loss of signal, failed registration attempts). |
| [R-6.15.4-011] | The MCX Service shall provide recording and audit functions for all types of MCX Service group communication (e.g., normal group, regroup group, Ad hoc group). |

# A.2	Requirements for off-network recording and audit

Requirements for off-network recording and audit in 3GPP TS 22.280 [8] are listed in Table A.2-1. They are applied to all services MCPTT, MCVideo and MCData according to the reference tables in Annexes A, B and C of 3GPP TS 22.280 [8].

Table A.2-1: Requirements for off-network recording and audit

| Requirement | Description |
| --- | --- |
| [R-7.14-001] | The Off-Network MCX Service shall provide a mechanism to collect metadata for MCX Service Group Communications and MCX Service Private Communications (e.g., initiating MCX Service User ID, MCX Service Group ID) and non-communication activities (e.g., changing group settings) from MCX UEs operating in off-network mode. Metadata shall be logged for both the transmitting Participant and the receiving Participant(s). |
| [R-7.14-001a] | The Off-Network MCX Service shall provide a mechanism for a Mission Critical Organization to record the media content of the transmitting Participant of Group Communications and Private Communications under the organization's authority from MCX UEs operating in off-network mode. NOTE: The handling of storage limitation of the UE is out of scope of 3GPP. |
| [R-7.14-002] | Upon return to on-network operation, the MCX Service shall provide a mechanism to retrieve communication and non-communication activity metadata from an MCX UE that has collected such metadata while operating in off-network mode. |
| [R-7.14-002a] | Upon return to on-network operation, the MCX Service shall provide a mechanism to retrieve the media content of the transmitting Participant of Group Communications and Private Communications from an MCX UE that has recorded such media content while operating in off-network mode. |

Annex B (informative):
Mapping of stage 1 requirements to solutions

| Requirement | Scenarios | Key Issues | Solutions | NOTE |
| --- | --- | --- | --- | --- |

Annex C (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-04 | SA6#66 | S6-251269 |  |  |  | Skeleton | 0.0.0 |
| 2025-04 | SA6#66 | S6-251270 S6-251271 S6-251272 |  |  |  | Scope References and Annex A Terms | 0.1.0 |
| 2025-05 | SA6#67 | S6-252330 |  |  |  | Restructuring the TR | 0.2.0 |
| 2025-05 | SA6#67 | S6-252320 |  |  |  | Scenario: Recording MCVideo one to one pull and push | 0.2.0 |
| 2025-05 | SA6#67 | S6-252321 |  |  |  | Key Issue: Recording MCVideo one to one pull and push | 0.2.0 |
| 2025-05 | SA6#67 | S6-252322 |  |  |  | Solution: Recording MCVideo one to one pull and push NOTE: content of S6-252322 adapted to the clause 6 structure. | 0.2.0 |
| 2025-05 | SA6#67 | S6-252323 |  |  |  | Key Issue: Setting recording targets for MCData | 0.2.0 |
| 2025-05 | SA6#67 | S6-252324 |  |  |  | Solution: Setting recording targets for MCData | 0.2.0 |
| 2025-05 | SA6#67 | S6-252325 |  |  |  | Key Issue: Both parties of a private call or one-to-one session as recording targets | 0.2.0 |
| 2025-05 | SA6#67 | S6-252326 |  |  |  | Key Issue: Recording and audit of e2e encrypted one-to-one communication | 0.2.0 |
| 2025-05 | SA6#67 | S6-252327 |  |  |  | Key Issue: Recording and audit of e2e encrypted group communication | 0.2.0 |
| 2025-09 | SA6#68 | S6-253431 |  |  |  | Scenarios – General | 0.3.0 |
| 2025-09 | SA6#68 | S6-253432 |  |  |  | Scenario 1 – Recording SIP session based communications | 0.3.0 |
| 2025-09 | SA6#68 | S6-253433 |  |  |  | Scenario 2 – Recording non-session-based communications | 0.3.0 |
| 2025-09 | SA6#68 | S6-253434 |  |  |  | Scenario 3 - Recording non-communication related user activities | 0.3.0 |
| 2025-09 | SA6#68 | S6-253042 |  |  |  | Scenario 4 - Recording during interconnection and migration | 0.3.0 |
| 2025-09 | SA6#68 | S6-253043 |  |  |  | Scenario 5 - Recording off-network communications | 0.3.0 |
| 2025-09 | SA6#68 | S6-253437 |  |  |  | Key Issue: Recording MC Data MSRP sessions | 0.3.0 |
| 2025-09 | SA6#68 | S6-253438 |  |  |  | Key Issue: Recording HTTP traffic | 0.3.0 |
| 2025-09 | SA6#68 | S6-253439 |  |  |  | Key Issue – Recording SDS over signalling control plane | 0.3.0 |
| 2025-09 | SA6#68 | S6-253440 |  |  |  | Key issue for supporting recording of non-communication related user activities for MC services | 0.3.0 |
| 2025-09 | SA6#68 | S6-253442 |  |  |  | Solution: Recording MC Data MSRP sessions | 0.3.0 |
| 2025-09 | SA6#68 | S6-253214 |  |  |  | Solution to KI#3 – both parties as recording targets | 0.3.0 |
| 2025-09 | SA6#68 | S6-253054 |  |  |  | Updating Annex B - the mapping table | 0.3.0 |